Annex F (normative):
Measurement uncertainties and Test Tolerances

# F.1	Acceptable uncertainty of Test System (normative)

The maximum acceptable uncertainty of the Test System is specified below for each test, where appropriate. The Test System shall enable the stimulus signals in the test case to be adjusted to within the specified range, and the equipment under test to be measured with an uncertainty not exceeding the specified values. All ranges and uncertainties are absolute values, and are valid for a confidence level of 95 %, unless otherwise stated.

A confidence level of 95 % is the measurement uncertainty tolerance interval for a specific measurement that contains 95 % of the performance of a population of test equipment.

For RF tests it should be noted that the uncertainties in clause F.1 apply to the Test System operating into a nominal 50 ohm load and do not include system effects due to mismatch between the DUT and the Test System.

The downlink signal uncertainties apply at each receiver antenna connector.

## F.1.1	Measurement of test environments

The measurement accuracy of the UE test environments defined in TS 36.508 subclause 4.1, Test environments shall be.

-	Pressure				5 kPa.

-	Temperature			2 degrees.

-	Relative Humidity	5 %.

-	DC Voltage			1,0 %.

-	AC Voltage			1,5 %.

-	Vibration				10 %.

-	Vibration frequency	0,1 Hz.

The above values shall apply unless the test environment is otherwise controlled and the specification for the control of the test environment specifies the uncertainty for the parameter.

## F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty for transmitter tests

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2.2 UE Maximum Output Power | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.3 dB, 4.2GHz < f ≤ 6GHz |  |
| 6.2.2_1 Maximum Output Power for HPUE | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.2.2A.1 UE Maximum Output Power for CA (intra-band contiguous DL CA and UL CA) | Same as 6.2.2 for sum of powers of all CCs |  |
| 6.2.2A.2 UE Maximum Output Power for CA (inter-band DL CA and UL CA) | Same as 6.2.2 for sum of powers of all CCs with f as the higher one among centre frequencies of 2 CCs. | Overall system uncertainty is maximum of UL measurement uncertainty of 2 CCs. |
| 6.2.2A.4 UE Maximum Output Power for CA (3UL CA) | For UL CA_XD: For BWagg ≤ 40MHz Same as 6.2.2 for sum of powers of all CCs For 40MHz < BWagg ≤ 60MHz ±1.4 dB for f ≤ 4.2GHz ±1.6 dB for 4.2GHz < f ≤ 6.0GHz For UL CA_XC_YA: Same as 6.2.2 for sum of powers of all CCs with f as the higher one among centre frequencies of XC or YA. For UL CA_XA_YA_ZA: FFS | Overall system uncertainty is maximum of UL measurement uncertainty of interband or non-contiguous CCs |
| 6.2.2B UE Maximum Output Power for UL-MIMO | Same as 6.2.2 for each antenna |  |
| 6.2.2B_1 HPUE Maximum Output Power for UL-MIMO | Same as 6.2.2_1 for each antenna |  |
| 6.2.2E UE Maximum Output Power for UE category 0 | Same as 6.2.2 |  |
| 6.2.2EA UE Maximum Output Power for UE category M1 | Same as 6.2.2 |  |
| 6.2.2EB UE Maximum Output Power for UE category 1bis | Same as 6.2.2 |  |
| 6.2.2EC UE Maximum Output Power for UE category M2 | Same as 6.2.2 |  |
| 6.2.2F UE Maximum Output Power for UE category NB1 and NB2 | Same as 6.2.2_1 |  |
| 6.2.2FA UE Maximum Output Power for category NB1 and NB2/Power Class 6 | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.2.2G.1 UE maximum output power for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Same as 6.2.2 |  |
| 6.2.2G.2 UE maximum output power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission | Same as 6.2.2 for sum of powers of all CCs |  |
| 6.2.2G.3 UE maximum output power for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 6.2.2 for sum of powers of all CCs |  |
| 6.2.2G.4.1 UE Maximum Output Power for V2X Communication / Power class 2 / Non-concurrent with E-UTRA uplink transmissions | Same as 6.2.2 |  |
| 6.2.2_s UE Maximum Output Power for subslot/slot TTI | Same as 6.2.2 |  |
| 6.2.3 Maximum Power Reduction | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.3 dB, 4.2GHz < f ≤ 6GHz |  |
| 6.2.3_1 Maximum Power Reduction (MPR) for HPUE | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.2.3A.1 Maximum Power Reduction (MPR) for CA (intra-band contiguous DL CA and UL CA) | Same as 6.2.3 for sum of powers of all CCs |  |
| 6.2.3A.1_1 Maximum Power Reduction (MPR) for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM | Same as 6.2.3 for sum of powers of all CCs |  |
| 6.2.3A.1_2 Maximum Power Reduction (MPR) for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM | Same as 6.2.3 for sum of powers of all CCs |  |
| 6.2.3A.2 Maximum Power Reduction (MPR) for CA (inter-band DL CA and UL CA) | Same as 6.2.2A.2 |  |
| 6.2.3A.2_1 Maximum Power Reduction (MPR) for CA (inter-band DL CA and UL CA) for UL 64QAM | Same as 6.2.2A.2 |  |
| 6.2.3A.2_2 Maximum Power Reduction (MPR) for CA (inter-band DL CA and UL CA) for UL 256QAM | Same as 6.2.3 for sum of powers of all CCs |  |
| 6.2.3A.3 Maximum Power Reduction (MPR) for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.2.2A.2 |  |
| 6.2.3A.3_1 Maximum Power Reduction (MPR) for CA (intra-band non-contiguous DL CA and UL CA) for UL 64QAM | Same as 6.2.2A.2 |  |
| 6.2.3A.3_2 Maximum Power Reduction (MPR) for CA (intra-band non-contiguous DL CA and UL CA) for UL 256QAM | Same as 6.2.2A.2 |  |
| 6.2.3A.4 Maximum Power Reduction (MPR) for CA (3UL CA) | Same as 6.2.3 for sum of powers of all CCs |  |
| 6.2.3B Maximum Power Reduction (MPR) for UL-MIMO | Same as 6.2.3 for each antenna |  |
| 6.2.3_2 Maximum Power Reduction (MPR) for Multi-Cluster PUSCH | Same as 6.2.3 |  |
| 6.2.3E Maximum Power Reduction for UE category 0 | Same as 6.2.3 |  |
| 6.2.3EA Maximum Power Reduction for UE category M1 | Same as 6.2.3 |  |
| 6.2.3EB Maximum Power Reduction for UE category 1bis | Same as 6.2.3 |  |
| 6.2.3_3 Maximum Power Reduction (MPR) for UL 64QAM | Same as 6.2.3 |  |
| 6.2.3_4 Maximum Power Reduction (MPR) for Multi-Cluster PUSCH with UL 64QAM | Same as 6.2.3 |  |
| 6.2.3F Maximum Power Reduction (MPR) for category NB1 and NB2 | Same as 6.2.3 |  |
| 6.2.3G.1.1 Maximum Power Reduction (MPR) for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Same as 6.2.3 |  |
| 6.2.3G.1.2 Maximum Power Reduction (MPR) for V2X Communication / Power class 3 / Non-contiguous allocation of PSCCH and PSSCH / Non-concurrent with E-UTRA uplink transmissions | Same as 6.2.3 |  |
| 6.2.3G.1.3 Maximum Power Reduction (MPR) for V2X Communication / Power class 3 / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission | Same as 6.2.3 for each CC |  |
| 6.2.3G.1.4 Maximum Power Reduction (MPR) for V2X Communication / Power class 3 / Intra-band contiguous multi-carrier operation | Same as 6.2.3 for sum of power |  |
| 6.2.3G.2.1 Maximum Power Reduction (MPR) for V2X Communication / Power class 2 / Contiguous allocation of PSCCH and PSSCH / Non-concurrent with E-UTRA uplink transmissions | Same as 6.2.3 |  |
| 6.2.3G.2.2 Maximum Power Reduction (MPR) for V2X Communication / Power class 2 / Non-contiguous allocation of PSCCH and PSSCH / Non-concurrent with E-UTRA uplink transmissions | Same as 6.2.3 |  |
| 6.2.3_5 Maximum Power Reduction (MPR) for UL 256QAM | Same as 6.2.3 |  |
| 6.2.3_6 Maximum Power Reduction (MPR) for Multi-Cluster PUSCH with UL 256QAM | Same as 6.2.3 |  |
| 6.2.3_s Maximum Power Reduction (MPR) for subslot/slot TTI | Same as 6.2.3 |  |
| 6.2.4 UE Maximum Output Power with additional requirements | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.2.4_1 Additional Maximum Power Reduction (A-MPR) for HPUE | ±0.7 dB, f ≤ 3.0GHz |  |
| 6.2.4_2 Additional Maximum Power Reduction (A-MPR) for UL 64QAM | Same as 6.2.4 |  |
| 6.2.4_3 Additional Maximum Power Reduction (A-MPR) with PUSCH frequency hopping | Same as 6.2.4 |  |
| 6.2.4_4 Additional Maximum Power Reduction (A-MPR) for UL 256QAM | Same as 6.2.4 |  |
| 6.2.4A.1 Additional Maximum Power Reduction (A-MPR) for CA (intra-band contiguous DL CA and UL CA) | Same as 6.2.4 for sum of powers of all CCs |  |
| 6.2.4A.1_1 Additional Maximum Power Reduction (A-MPR) for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM | Same as 6.2.4 for sum of powers of all CCs |  |
| 6.2.4A.1_2 Additional Maximum Power Reduction (A-MPR) for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM | Same as 6.2.4 for sum of powers of all CCs |  |
| 6.2.4A.2 Additional Maximum Power Reduction (A-MPR) for CA (inter-band DL CA and UL CA) | Same as 6.2.2A.2 |  |
| 6.2.4A.2_1 Additional Maximum Power Reduction (A-MPR) for CA (inter-band DL CA and UL CA) for UL 64QAM | Same as 6.2.2A.2 |  |
| 6.2.4A.2_2 Additional Maximum Power Reduction (A-MPR) for CA (inter-band DL CA and UL CA) for UL 256QAM | Same as 6.2.2A.2 |  |
| 6.2.4A.4 Additional Maximum Power Reduction (A-MPR) for CA (3UL CA) | Same as 6.2.4 for sum of powers of all CCs |  |
| 6.2.4B Additional Maximum Power Reduction (A-MPR) for UL-MIMO | Same as 6.2.4 for each antenna |  |
| 6.2.4E Additional Maximum Power Reduction (A-MPR) for UE category 0 | Same as 6.2.4 |  |
| 6.2.4EA Additional Maximum Power Reduction (A-MPR) for UE category M1 | Same as 6.2.4 |  |
| 6.2.4EB Additional Maximum Power Reduction (A-MPR) for UE category 1bis | Same as 6.2.4 |  |
| 6.2.4EC Additional Maximum Power Reduction (A-MPR) for UE category M2 | Same as 6.2.4EA |  |
| 6.2.4G.1 Additional Maximum Power Reduction (A-MPR) for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | ±1.3 dB, 4.2GHz < f ≤ 6GHz |  |
| 6.2.4G.2 Additional Maximum Power Reduction (A-MPR) for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | ±1.3 dB for sum of power | Maximum MU within both carriers. |
| 6.2.4_s Additional Maximum Power Reduction (A-MPR) for subslot/slot TTI | Same as 6.2.4 |  |
| 6.2.5 Configured UE transmitted Output Power | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.2.5_s Configured UE Transmitted Output Power for subslot/slot TTI | Same as 6.2.5 |  |
| 6.2.5_1 Configured UE transmitted Output Power for HPUE | Same as 6.2.5 |  |
| 6.2.5A.1 Configured UE transmitted Output Power for CA (intra-band contiguous DL CA and UL CA) | Same as 6.2.5 for sum of powers of all CCs |  |
| 6.2.5A.1_3 Configured UE transmitted Output Power for CA and HPUE (intra-band contiguous DL CA and UL CA) for HPUE | Same as 6.2.5 for sum of powers of all CCs |  |
| 6.2.5A.3 Configured UE transmitted Output Power for CA (inter-band DL CA and UL CA) | Same as 6.2.2A.2 |  |
| 6.2.5A.4 Configured UE transmitted Output Power for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.2.2A.2 |  |
| 6.2.5B Configured UE transmitted output power for UL-MIMO | Same as 6.2.5 for each antenna |  |
| 6.2.5E Configured Transmitted Power for UE category 0 | Same as 6.2.5 |  |
| 6.2.5EA Configured UE transmitted Power for UE category M1 | Same as 6.2.5 |  |
| 6.2.5EB Configured UE transmitted Power for UE category 1bis | Same as 6.2.5 |  |
| 6.2.5F Configured UE transmitted Output Power for UE category NB1 and NB2 | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.2.5G.1 Configured UE transmitted Output Power for V2X Communication / Non-concurrent with E-UTRA uplink transmission | ±1.3 dB, 4.2GHz < f ≤ 6.0GHz, for UE powers >= 0dBm ±1.5 dB, 4.2GHz < f ≤ 6.0GHz, for -40 dBm <= UE powers <0dBm |  |
| 6.2.5G.2 Configured UE transmitted Output Power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission | ±1.3 dB, 4.2GHz < f ≤ 6.0GHz, for UE powers >= 0dBm ±1.5 dB, 4.2GHz < f ≤ 6.0GHz, for -40 dBm <= UE powers <0dBm |  |
| 6.2.5G.3 Configured UE transmitted Output Power for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 6.2.5G.1 for each CC |  |
| 6.3.2 Minimum Output Power | ±1.0 dB, f ≤ 3.0GHz ±1.3 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 6GHz |  |
| 6.3.2A.1 Minimum Output Power for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.2 for each CC |  |
| 6.3.2A.4 Minimum Output Power for CA (3UL CA) | Same as 6.3.2 for each CC |  |
| 6.3.2B Minimum Output Power for UL-MIMO | Same as 6.3.2 for each antenna |  |
| 6.3.2E Minimum Output Power for UE category 0 | Same as 6.3.2 |  |
| 6.3.2EA Minimum Output Power for UE category M1 | Same as 6.3.2 |  |
| 6.3.2EB Minimum Output Power for UE category 1bis | Same as 6.3.2 |  |
| 6.3.2F Minimum Output Power for UE category NB1 and NB2 | ±1.0 dB, f ≤ 3.0GHz ±1.3 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.3.2G.1 Minimum output power for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Same as 6.3.2 |  |
| 6.3.2G.2 Minimum output power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | Same as 6.3.2 |  |
| 6.3.2G.3 Minimum output power for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 6.3.2 |  |
| 6.3.3 Transmission ON/OFF Power | Transmission OFF Power: ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz ±2.0 dB, 4.2GHz < f ≤ 6GHz |  |
| 6.3.3A.1 UE Transmit OFF power for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.3 for each CC |  |
| 6.3.3A.2 UE Transmit OFF power for CA (inter-band DL CA and UL CA) | Same as 6.3.3A.1 |  |
| 6.3.3A.3 UE Transmit OFF power for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.3.3 for each CC |  |
| 6.3.3A.4 UE Transmit OFF power for CA (3UL CA) | Same as 6.3.3 for each CC |  |
| 6.3.3B UE Transmit OFF power for UL-MIMO | Same as 6.3.3 for each antenna |  |
| 6.3.3D UE Transmit OFF power for ProSe | Same as 6.3.3 |  |
| 6.3.3E UE Transmit OFF Power for UE category 0 | Same as 6.3.3 |  |
| 6.3.3EA UE Transmit OFF power for UE category M1 | Same as 6.3.3 |  |
| 6.3.3EB UE Transmit OFF power for UE category 1bis | Same as 6.3.3 |  |
| 6.3.3F Transmit OFF power for category NB1 and NB2 | Same as 6.3.3 |  |
| 6.3.3G.1 UE Transmit OFF power for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Same as 6.3.3 |  |
| 6.3.3G.2 UE Transmit OFF power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | Same as 6.3.3 for each CC |  |
| 6.3.3G.3 UE Transmit OFF power for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 6.3.3 for each CC |  |
| 6.3.4.1 General ON/OFF time mask | Transmission ON/OFF Power: ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.3.4.1_s General ON/OFF time mask for subslot/slot TTI | Same as 6.3.4.1 |  |
| 6.3.4.2 PRACH and SRS time mask | Transmission ON/OFF Power: ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.3.4A.1.2 General ON/OFF time mask for CA (inter-band DL CA and UL CA) | Same as 6.3.4A.1.1 |  |
| 6.3.4A.1.1 General ON/OFF time mask for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.4.1 for each CC |  |
| 6.3.4A.1.3 General ON/OFF time mask for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.3.4.1 for each CC |  |
| 6.3.4A.1.4 General ON/OFF time mask for CA (3UL CA) | Same as 6.3.4.1 for each CC |  |
| 6.3.4A.1.5 General ON/OFF time mask for CA (4UL CA) | Same as 6.3.4.1 for each CC |  |
| 6.3.4B.1 General ON/OFF time mask for UL-MIMO | Same as 6.3.4.1 for each antenna |  |
| 6.3.4C.1 General ON/OFF time mask for Dual Connectivity | Same as 6.3.4A.1.1 |  |
| 6.3.4E.1 General ON/OFF time mask for UE category 0 | Same as 6.3.4.1 |  |
| 6.3.4E.2 PRACH and SRS time mask for UE category 0 | Same as 6.3.4.2 |  |
| 6.3.4EA.1 General ON/OFF time mask for UE category M1 | Same as 6.3.4.1 |  |
| 6.3.4EA.2 PRACH and SRS ON/OFF time mask for UE category M1 | Same as 6.3.4.2 |  |
| 6.3.4EB.1 General ON/OFF time mask for UE category 1bis | Same as 6.3.4.1 |  |
| 6.3.4EB.2 PRACH and SRS ON/OFF time mask for UE category 1bis | Same as 6.3.4.2 |  |
| 6.3.4EC.2 PRACH and SRS ON/OFF time mask for UE category M2 | Same as 6.3.4.2 |  |
| 6.3.4F.1 General ON/OFF time mask for category NB1 and NB2 | Transmission ON/OFF Power: ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.3.4F.2 NPRACH time mask for category NB1 and NB2 | Transmission ON/OFF Power: ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.3.4G.1 General ON/OFF time mask for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | ±2.0 dB, 4.2GHz < f ≤ 6GHz |  |
| 6.3.4G.2 General ON/OFF time mask for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | Transmission ON/OFF Power: ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz ±2.0 dB, 4.2GHz < f ≤ 6.0GHz |  |
| 6.3.4G.3 General ON/OFF time mask for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 6.3.4G.1 for each CC |  |
| 6.3.4G.4 PSSS/SSSS time mask for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | ±2.0 dB, 4.2GHz < f ≤ 6GHz |  |
| 6.3.5.1 Power Control Absolute power tolerance | ±1.0 dB, f ≤ 3.0GHz ±1.4 dB, 3.0GHz < f ≤ 4.2GHz ±2.0 dB, 4.2GHz < f ≤ 6GHz | Overall system uncertainty comprises two quantities: 1. Downlink signal level uncertainty 2. Uplink level measurement uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared. Test System uncertainty = [SQRT (DL level uncert 2 + UL measurement uncert 2)] f ≤ 3.0GHz DL signal level uncert ± 0.7dB UL meas’t uncert ± 0.7dB 3.0GHz < f ≤ 4.2GHz DL signal level uncert ± 1.0dB UL meas’t uncert ± 1.0dB 4.2GHz < f ≤ 6GHz DL signal level uncert ± 1.5dB UL meas’t uncert ± 1.3dB |
| 6.3.5.2 Power Control Relative power tolerance | ±0.7 dB |  |
| 6.3.5.3 Aggregate power control tolerance | ±0.7 dB |  |
| 6.3.5_1.1 Power Control Absolute power tolerance for HPUE | ±1.0 dB, f ≤ 3.0GHz ±1.4 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.3.5_1.2 Power Control Relative power tolerance for HPUE | ±0.7 dB |  |
| 6.3.5_1.3 Aggregate power control tolerance for HPUE | ±0.7 dB |  |
| 6.3.5A.1.1 Power Control Absolute power tolerance for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.5.1 for each CC |  |
| 6.3.5A.1.2 Power Control Absolute power tolerance for CA (inter-band DL CA and UL CA) | Same as 6.3.5.1 for each CC |  |
| 6.3.5A.1.3 Power Control Absolute power tolerance for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.3.5.1 for each CC |  |
| 6.3.5A.1.4 Power Control Absolute power tolerance for CA (3UL CA) | Same as 6.3.5.1 for each CC |  |
| 6.3.5A.1.5 Power Control Absolute power tolerance for CA(SRS carrier based switching) | Same as 6.3.5.1 for SCC |  |
| 6.3.5A.1.6 Power Control Absolute power tolerance for CA (4UL CA) | Same as 6.3.5.1 for each CC |  |
| 6.3.5A.2.1 Power Control Relative power tolerance for CA (intra-band contiguous DL CA and UL CA) | Absolute power ±1dB, f ≤ 4.2GHz Relative power ±0.7 dB Both requirements apply for each CC | Absolute power is used to check when power is near lower and upper limits of the range. Relative power is used to check step sizes. |
| 6.3.5A.2.4.1 Power Control Relative power tolerance for CA (intra-band contiguous 3DL CA and 3UL CA) | Same as 6.3.5A.2.1 |  |
| 6.3.5A.2.4.2 Power Control Relative power tolerance for CA (inter-band 3DL CA and 3UL CA) | Same as 6.3.5A.2.2 |  |
| 6.3.5A.2.4.3 Power Control Relative power tolerance for CA (intra-band non-contiguous 3DL CA and 3UL CA) | Same as 6.3.5A.2.3 |  |
| 6.3.5A.3.1 Aggregate power control tolerance for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.5.3 for each CC |  |
| 6.3.5A.3.2 Aggregate power control tolerance for CA (inter-band DL CA and UL CA) | Same as 6.3.5.3 for each CC |  |
| 6.3.5A.3.3 Aggregate power control tolerance for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.3.5.3 for each CC |  |
| 6.3.5A.3.4 Aggregate Power Control Tolerance for CA (3UL CA) | Same as 6.3.5.3 for each CC |  |
| 6.3.5A.3.5 Aggregate Power Control Tolerance for CA (4UL CA) | Same as 6.3.5.3 for each CC |  |
| 6.3.5B.1 Power Control Absolute Power Tolerance for UL- MIMO | Same as 6.3.5.1 for each antenna |  |
| 6.3.5B.2 Power Control Relative power tolerance for UL-MIMO | Same as 6.3.5.2 for each antenna |  |
| 6.3.5B.3 Aggregate power control tolerance for UL-MIMO | Same as 6.3.5.3 for each antenna |  |
| 6.3.5C.2 Power Control Relative power tolerance for Dual Connectivity | Same as 6.3.5A.2.1 |  |
| 6.3.5C.2_1 Power Control Relative power tolerance for asynchronous Dual Connectivity | Same as 6.3.5A.2.1 |  |
| 6.3.5E.1 Power Control Absolute power tolerance for UE category 0 | Same as 6.3.5.1 |  |
| 6.3.5E.2 Power Control Relative power tolerance for UE category 0 | Same as 6.3.5.2 |  |
| 6.3.5E.3 Aggregate power control tolerance for UE category 0 | Same as 6.3.5.3 |  |
| 6.3.5EA.1 Power Control Absolute power tolerance for UE category M1 | ±1.0 dB, f ≤ 3.0GHz ±1.4 dB, 3.0GHz < f ≤ 4.2GHz | Overall system uncertainty comprises two quantities: 1. Downlink signal level uncertainty 2. Uplink level measurement uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared. Test System uncertainty = [SQRT (DL level uncert 2 + UL measurement uncert 2)] f ≤ 3.0GHz DL signal level uncert ± 0.7dB UL meas’t uncert ± 0.7dB 3.0GHz < f ≤ 4.2GHz DL signal level uncert ± 1.0dB UL meas’t uncert ± 1.0dB |
| 6.3.5EA.2 Power Control Relative power tolerance for UE category M1 | ±0.7 dB |  |
| 6.3.5EA.3 Aggregate power control tolerance for UE category M1 | Same as 6.3.5.3 |  |
| 6.3.5EA.3_1 Aggregate power control tolerance for UE category M1 (CE Mode B) | Same as 6.3.5.3 |  |
| 6.3.5EB.1 Power Control Absolute power tolerance for UE category 1bis | Same as 6.3.5.1 |  |
| 6.3.5EC.3_1 Aggregate power control tolerance for UE category M2 (CE Mode B) | Same as 6.3.5.1 |  |
| 6.3.5F.1 Power Control Absolute power tolerance for category NB1 and NB2 | ±1.0 dB, f ≤ 3.0GHz ±1.4 dB, 3.0GHz < f ≤ 4.2GHz | Overall system uncertainty comprises two quantities: 1. Downlink signal level uncertainty 2. Uplink level measurement uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared. Test System uncertainty = [SQRT (DL level uncert 2 + UL measurement uncert 2)] f ≤ 3.0GHz DL signal level uncert ± 0.7dB UL meas’t uncert ± 0.7dB 3.0GHz < f ≤ 4.2GHz DL signal level uncert ± 1.0dB UL meas’t uncert ± 1.0dB |
| 6.3.5F.2 Power Control Relative power tolerance for category NB1 and NB2 | ±0.7 dB |  |
| 6.3.5F.3 Aggregate power control tolerance for category NB1 and NB2 | ±0.7 dB |  |
| 6.3.5G.1 Power Control Absolute power tolerance for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | ±2.0 dB, 4.2GHz < f ≤ 6GHz | Overall system uncertainty comprises two quantities: 1. Downlink signal level uncertainty 2. Uplink level measurement uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared. Test System uncertainty = [SQRT (DL level uncert 2 + UL measurement uncert 2)] 4.2GHz < f ≤ 6GHz DL signal level uncert ± 1.5dB UL meas’t uncert ± 1.3dB |
| 6.3.5G.2 Power Control Absolute power tolerance for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | ±1.0 dB, f ≤ 3.0GHz ±2.0 dB, 4.2GHz < f ≤ 6GHz | Overall system uncertainty comprises two quantities: 1. Downlink signal level uncertainty 2. Uplink level measurement uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared. Test System uncertainty = [SQRT (DL level uncert 2 + UL measurement uncert 2)] f ≤ 3.0GHz DL signal level uncert ± 0.7dB UL meas’t uncert ± 0.7dB 4.2GHz < f ≤ 6GHz DL signal level uncert ± 1.5dB UL meas’t uncert ± 1.3dB |
| 6.3.5G.3 Power Control Absolute power tolerance for V2X Communication / Intra-band contiguous multi-carrier operation | ±2.0 dB, 4.2GHz < f ≤ 6GHz | Overall system uncertainty comprises two quantities: 1. Downlink signal level uncertainty 2. Uplink level measurement uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared. Test System uncertainty = [SQRT (DL level uncert 2 + UL measurement uncert 2)] 4.2GHz < f ≤ 6GHz DL signal level uncert ± 1.5dB UL meas’t uncert ± 1.3dB |
| 6.5.1 Frequency Error | ±15 Hz DL Signal level: ±0.7 dB, f ≤ 3.0GHz DL Signal level: ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.5.1A.1 Frequency error for CA (intra-band contiguous DL CA and UL CA) | Same as 6.5.1 for each CC |  |
| 6.5.1A.2 Frequency error for CA (inter-band DL CA and UL CA) | Same as 6.5.1 |  |
| 6.5.1A.3 Frequency error for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.5.1 for each CC |  |
| 6.5.1A.4 Frequency error for CA (3UL CA) | Same as 6.5.1 for each CC |  |
| 6.5.1A.5 Frequency error for CA (4UL CA) | Same as 6.5.1 for each CC |  |
| 6.5.1B Frequency Error for UL-MIMO | Same as 6.5.1 for each antenna DL signal level same as 6.5.1 |  |
| 6.5.1D.1 Frequency Error for ProSe Direct Discovery | ±15 Hz Sidelink Signal level: ±0.7 dB, f ≤ 3.0GHz |  |
| 6.5.1D.2 Frequency Error for ProSe Direct Communication | Same as 6.5.1D.1 |  |
| 6.5.1E Frequency Error for UE category 0 | Same as 6.5.1 |  |
| 6.5.1EA Frequency Error for UE category M1 | Same as 6.5.1 |  |
| 6.5.1EB Frequency Error for UE category 1bis | Same as 6.5.1 |  |
| 6.5.1EC Frequency Error for UE category M2 | Same as 6.5.1 |  |
| 6.5.1EC_1 Frequency Error for UE category M2 (CE Mode B) | Same as 6.5.1 |  |
| 6.5.1F Frequency Error for category NB1 | ±15 Hz DL Signal level: ±0.7 dB, f ≤ 3.0GHz |  |
| 6.5.1G.1 Frequency Error for V2X Communication / Non-concurrent with E-UTRA uplink transmission | ±36 Hz, f > 3.0GHz |  |
| 6.5.1G.2 Frequency error for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | ±15 Hz, f ≤ 3.0GHz ±36 Hz, 4.2GHz <f ≤ 6.0GHz |  |
| 6.5.2.1 Error Vector Magnitude | PUSCH: ± 2.5% PUCCH: ± 2.5% PRACH: ± 2.5% |  |
| 6.5.2.1_1 Error Vector Magnitude (EVM) for UL 64QAM | Same as 6.5.2.1 |  |
| 6.5.2.1_2 Error Vector Magnitude (EVM) for UL 256QAM | Same as 6.5.2.1 |  |
| 6.5.2A.1.1 Error Vector Magnitude (EVM) for CA (intra-band contiguous DL CA and UL CA) | Same as 6.5.2.1 |  |
| 6.5.2A.1.1_2 EVM for CA (intra-band contiguous DL CA and UL CA) with UL 256QAM | Same as 6.5.2.1 |  |
| 6.5.2A.1.2 Error Vector Magnitude (EVM) for CA (inter-band DL CA and UL CA) | Same as 6.5.2.1 |  |
| 6.5.2A.1.2_1 EVM for CA (inter-band DL CA and UL CA) with UL 64QAM | Same as 6.5.2.1 |  |
| 6.5.2A.1.2_2 Error Vector Magnitude (EVM) for CA (inter-band DL CA and UL CA) for UL 256QAM | Same as 6.5.2.1 |  |
| 6.5.2A.1.3 Error Vector Magnitude (EVM) for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.5.2.1 |  |
| 6.5.2A.1.1_1 EVM for CA (intra-band contiguous DL CA and UL CA) with UL 64QAM | Same as 6.5.2.1 |  |
| 6.5.2A.1.3_1 Error Vector Magnitude (EVM) for CA (intra-band non-contiguous DL CA and UL CA) for UL 64QAM | Same as 6.5.2.1 |  |
| 6.5.2A.1.3_2 Error Vector Magnitude (EVM) for CA (intra-band non-contiguous DL CA and UL CA) for UL 256QAM | Same as 6.5.2.1 |  |
| 6.5.2A.1.4 Error Vector Magnitude (EVM) for CA (3UL CA) | Same as 6.5.2.1 |  |
| 6.5.2A.1.5 Error Vector Magnitude (EVM) for CA (4UL CA) | Same as 6.5.2.1 |  |
| 6.5.2B.1 Error Vector Magnitude (EVM) for UL- MIMO | Same as 6.5.2.1 for each antenna |  |
| 6.5.2.1A PUSCH-EVM with exclusion period | PUSCH : ± 2.5% |  |
| 6.5.2.1E.1 Error Vector Magnitude for UE category 0 | Same as 6.5.2.1 |  |
| 6.5.2.1E.2 PUSCH-EVM with exclusion period for UE category 0 | Same as 6.5.2.1A |  |
| 6.5.2.1EA.1 Error Vector Magnitude for UE category M1 | Same as 6.5.2.1 |  |
| 6.5.2.1EA.2 PUSCH-EVM with exclusion period for UE category M1 | Same as 6.5.2.1A |  |
| 6.5.2.1EB Error Vector Magnitude (EVM) for UE category 1bis | Same as 6.5.2.1 |  |
| 6.5.2.1EC.1 Error Vector Magnitude for UE category M2 | Same as 6.5.2.1 |  |
| 6.5.2.1EC.2 PUSCH-EVM with exclusion period for UE category M1 | Same as 6.5.2.1A |  |
| 6.5.2.1F.1 Error Vector Magnitude (EVM) for UE category NB1 | Same as 6.5.2.1 |  |
| 6.5.2.1G.1 Error Vector Magnitude (EVM) for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Same as 6.5.2.1 |  |
| 6.5.2.1G.2 Error Vector Magnitude (EVM) for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | ± 2.5% |  |
| 6.5.2.1G.3 Error Vector Magnitude (EVM) for V2X Communication / Intra-band contiguous multi-carrier operation | ± 2.5% |  |
| 6.5.2.1_s Error Vector Magnitude (EVM) for subslot/slot TTI | Same as 6.5.2 |  |
| 6.5.2.2 Carrier leakage | ±0.8dB |  |
| 6.5.2A.2.1 Carrier leakage for CA (intra-band contiguous DL CA and UL CA) | Same as 6.5.2.2 |  |
| 6.5.2A.2.2 Carrier leakage for CA (inter-band DL CA and UL CA) | ±0.8dB |  |
| 6.5.2A.2.3 Carrier leakage for CA (intra-band non-contiguous DL CA and UL CA) | ±0.8dB |  |
| 6.5.2A.2.4 Carrier leakage for CA (3UL CA) | ±0.8dB |  |
| 6.5.2A.2.5 Carrier leakage for CA (4UL CA) | ±0.8dB |  |
| 6.5.2.3 In-band emissions for non allocated RB | ±0.8dB |  |
| 6.5.2.4 EVM equalizer Spectrum flatness | ±1.4dB |  |
| 6.5.2A.3.1 In-band emissions for non allocated RB for CA (intra-band contiguous DL CA and UL CA) | Same as 6.5.2.3 |  |
| 6.5.2A.3.2 In-band emissions for non allocated RB for CA (inter-band DL CA and UL CA) | Same as 6.5.2.3 for each CC |  |
| 6.5.2A.3.3 In-band emissions for non allocated RB for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.5.2.3 for each CC |  |
| 6.5.2A.3.4 In-band emissions for non allocated RB for CA (3UL CA) | Same as 6.5.2.3 for each CC |  |
| 6.5.2A.3.5 In-band emissions for non allocated RB for CA (4UL CA) | Same as 6.5.2.3 for each CC |  |
| 6.5.2B.1 Error vector magnitude (EVM) for UL-MIMO | TBD |  |
| 6.5.2B.2 Carrier leakage for UL-MIMO | Same as 6.5.2.2 for each antenna |  |
| 6.5.2B.3 In-band emissions for non allocated RB for UL-MIMO | Same as 6.5.2.3 for each antenna |  |
| 6.5.2B.4 EVM equalizer spectrum flatness for UL-MIMO | Same as 6.5.2.4 for each antenna |  |
| 6.5.2.2E Carrier Leakage for UE category 0 | Same as 6.5.2.2 |  |
| 6.5.2.2EA Carrier Leakage for UE category M1 | Same as 6.5.2.2 |  |
| 6.5.2.2EB Carrier leakage for UE category 1bis | Same as 6.5.2.2 |  |
| 6.5.2.2EC Carrier leakage for UE category M2 | Same as 6.5.2.2 |  |
| 6.5.2.2F Carrier leakage for UE category NB1 | Same as 6.5.2.2 |  |
| 6.5.2.2G.1 Carrier leakage for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Same as 6.5.2.2 |  |
| 6.5.2.2G.2 Carrier leakage for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission | Same as 6.5.2.2 |  |
| 6.5.2.2G.3 Carrier leakage for V2X Communication / Intra-band contiguous MCC operation | Same as 6.5.2.2 |  |
| 6.5.2.3E In-band emissions for non allocated RB for UE category 0 | Same as 6.5.2.3 |  |
| 6.5.2.3EA In-band emissions for non allocated RB for UE category M1 | Same as 6.5.2.3 |  |
| 6.5.2.3EB In-band emissions for non allocated RB for UE category 1bis | Same as 6.5.2.3 |  |
| 6.5.2.3EC In-band emissions for non allocated RB for UE category M2 | Same as 6.5.2.3 |  |
| 6.5.2.3F In-band emissions for non allocated RB for UE category NB1 | Same as 6.5.2.3 |  |
| 6.5.2.3G.1 In-band emissions for non-allocated RB for V2X Communication | Emission: ±0.8dB UE transmitted power ±1.3 dB, 4.2GHz < f ≤ 6.0GHz, for UE powers >= 0dBm ±1.5 dB, 4.2GHz < f ≤ 6.0GHz, for UE powers >= -40dBm |  |
| 6.5.2.3G.2 Introduction of In-band emissions for non-allocated RB for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | same as 6.5.2.3G.1 for E-UTRA uplink same as 6.5.2.3G.1 for E-UTRA V2X sidelink |  |
| 6.5.2.3G.3 In-band emissions for non-allocated RB for V2X Communication / Intra-band contiguous MCC operation | same as 6.5.2.3G.1 for cc of E-UTRA V2X sidelink |  |
| 6.5.2.4E EVM equalizer spectrum flatness for UE category 0 | Same as 6.5.2.4 |  |
| 6.5.2.4EA EVM equalizer spectrum flatness for UE category M1 | Same as 6.5.2.4 |  |
| 6.5.2.4EB EVM equalizer spectrum flatness for UE category 1bis | Same as 6.5.2.4 |  |
| 6.5.2.4EC EVM equalizer spectrum flatness for UE category M2 | Same as 6.5.2.4 |  |
| 6.5.2.4G.1 EVM equalizer spectrum flatness for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Same as 6.5.2.4 |  |
| 6.5.2.4G.3 EVM equalizer spectrum flatness for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 6.5.2.4 for each cc |  |
| 6.6.1 Occupied bandwidth | 1.4MHz, 3MHz: 30kHz 5MHz, 10MHz: 100kHz 15MHz, 20MHz: 300kHz |  |
| 6.6.1A.1 Occupied bandwidth for CA (intra-band contiguous DL CA and UL CA) | 1.4MHz, 3MHz: 30kHz 5MHz, 10MHz: 100kHz 15MHz, 20MHz: 300kHz 20MHz < f ≤ 40MHz: 500kHz |  |
| 6.6.1A.2 Occupied bandwidth for CA (inter-band DL CA and UL CA) | Same as 6.6.1 for each CC |  |
| 6.6.1A.3 Occupied bandwidth for CA (intra-band non-contiguous DL CA and UL CA) | 1.4MHz, 3MHz: 30kHz 5MHz, 10MHz: 100kHz 15MHz, 20MHz: 300kHz |  |
| 6.6.1A.5 Occupied bandwidth for CA (4UL CA) | 1.4MHz, 3MHz: 30kHz 5MHz, 10MHz: 100kHz 15MHz, 20MHz: 300kHz 20MHz < f ≤ 40MHz: 500kHz 40MHz < f ≤ 60MHz: [800kHz] 60MHz < f ≤ 80MHz: [1000kHz] for intra-band contiguous CA Same as 6.6.1 for each CC for inter-band CA |  |
| 6.6.1B Occupied bandwidth for UL-MIMO | Same as 6.6.1 for each antenna |  |
| 6.6.1E Occupied bandwidth for UE category 0 | Same as 6.6.1 |  |
| 6.6.1EA Occupied bandwidth for UE category M1 | Same as 6.6.1 |  |
| 6.6.1EB Occupied bandwidth for UE category 1bis | Same as 6.6.1 |  |
| 6.6.1EC Occupied bandwidth for UE category M2 | Same as 6.6.1 |  |
| 6.6.1G.1 Occupied bandwidth for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | Same as 6.6.1 |  |
| 6.6.1G.2 Occupied bandwidth for V2X Communication / Non-concurrent with E-UTRA uplink transmission | Same as 6.6.1 |  |
| 6.6.1G.3 Occupied bandwidth for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 6.6.1 |  |
| 6.6.1F Occupied bandwidth for UE category NB1 | 200kHz: 2kHz |  |
| 6.6.2.1 Spectrum Emission Mask | ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.6.2.1_1 Spectrum Emission Mask for Multi-Cluster PUSCH | Same as 6.6.2.1 |  |
| 6.6.2.1A.1 Spectrum emission mask for CA (intra-band contiguous DL CA and UL CA) | Same as 6.6.2.1 |  |
| 6.6.2.1A.2 Spectrum Emission Mask for CA (inter-band DL CA and UL CA) | Same as 6.6.2.1 and 6.6.2.2 |  |
| 6.6.2.1A.3 Spectrum Emission Mask for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.6.2.1 |  |
| 6.6.2.1A.5 Additional Spectrum Emission Mask for CA (4UL) | Same as 6.6.2.1 |  |
| 6.6.2.1B Spectrum Emission Mask for UL-MIMO | Same as 6.6.2.1 for each antenna |  |
| 6.6.2.1E Spectrum Emission Mask for UE category 0 | Same as 6.6.2.1 |  |
| 6.6.2.1EA Spectrum Emission Mask for UE category M1 | Same as 6.6.2.1 |  |
| 6.6.2.1EB Spectrum Emission Mask for UE category 1bis | Same as 6.6.2.1 |  |
| 6.6.2.1EC Spectrum Emission Mask for UE category M2 | Same as 6.6.2.1 |  |
| 6.6.2.1F Spectrum Emission Mask for UE category NB1 | Same as 6.6.2.1 |  |
| 6.6.2.1G.1 Spectrum Emission Mask for V2X Communication Non-concurrent with E-UTRA uplink transmissions | ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz ±2.0 dB, 4.2GHz < f ≤ 6GHz |  |
| 6.6.2.1G.2 Spectrum Emission Mask for V2X Communication /Sidelink simultaneous with E-UTRA uplink transmissions | same as 6.6.2.1G.1 for V2X Sidelink same as 6.6.2.1G.1 for E-UTRA Uplink |  |
| 6.6.2.1G.3 Spectrum Emission Mask for V2X Communication / Intra-band contiguous MCC operation | Same as 6.6.2.1G.1 |  |
| 6.6.2.2 Additional Spectrum Emission Mask | ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz ±2.0 dB, 4.2GHz < f ≤ 6.0GHz |  |
| 6.6.2.2_1 Additional Spectrum Emission Mask for UL 64QAM | Same as 6.6.2.2 |  |
| 6.6.2.2_2 Additional Spectrum Emission Mask for UL 256QAM | Same as 6.6.2.2 |  |
| 6.6.2.2A.1 Additional Spectrum Emission Mask for CA (intra-band contiguous DL CA and UL CA) | ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.6.2.2A.1_1 Additional Spectrum Emission Mask for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM | Same as 6.6.2.2A |  |
| 6.6.2.2A.1_2 Additional Spectrum Emission Mask for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM | Same as 6.6.2.2A |  |
| 6.6.2.2A.2 Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA) | Same as 6.6.2.2 |  |
| 6.6.2.2A.2_1 Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA) for UL 64QAM | Same as 6.6.2.2 |  |
| 6.6.2.2A.2_2 Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA) for UL 256QAM | Same as 6.6.2.2 |  |
| 6.6.2.2A.3 Additional Spectrum Emission Mask for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.6.2.2 |  |
| 6.6.2.2A.5 Additional Spectrum Emission Mask for CA (4UL CA) | Same as 6.6.2.2 |  |
| 6.6.2.2E Additional Spectrum Emission Mask for UE category 0 | Same as 6.6.2.2 |  |
| 6.6.2.2EA Additional Spectrum Emission Mask for UE category M1 | Same as 6.6.2.2 |  |
| 6.6.2.2EB Additional Spectrum Emission Mask for UE category 1bis | Same as 6.6.2.2 |  |
| 6.6.2.2EC Additional Spectrum Emission Mask for UE category M2 | Same as 6.6.2.2 |  |
| 6.6.2.2G.1 Additional Spectrum Emission Mask for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Same as 6.6.2.2 |  |
| 6.6.2.2G.2 Additional Spectrum Emission Mask for V2X Communication / Sidelink simultaneous with E-UTRA uplink transmissions | Same as 6.6.2.2 for V2X Sidelink Same as 6.6.2.2 E-UTRA uplink |  |
| 6.6.2.3_2 Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH | Same as 6.6.2.3 |  |
| 6.6.2.3 Adjacent Channel Leakage power Ratio | ±0.8 dB |  |
| 6.6.2.3_1 Adjacent Channel Leakage power Ratio for HPUE | ±0.8 dB |  |
| 6.6.2.3_2 Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH | Same as 6.6.2.3 |  |
| 6.6.2.3_3 Adjacent Channel Leakage power Ratio for UL 64QAM | Same as 6.6.2.3 |  |
| 6.6.2.3_4 Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH with UL 64QAM | Same as 6.6.2.3 |  |
| 6.6.2.3_5 Adjacent Channel Leakage power Ratio for UL 256QAM | Same as 6.6.2.3 |  |
| 6.6.2.3_6 Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH with UL 256QAM | Same as 6.6.2.3 |  |
| 6.6.2.3A.1 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) | ±0.8 dB |  |
| 6.6.2.3A.1_1 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM | Same as 6.6.2.3A.1 |  |
| 6.6.2.3A.1_2 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM | Same as 6.6.2.3A.1 |  |
| 6.6.2.3A.2 Adjacent Channel Leakage power Ratio for CA (inter-band DL CA and UL CA) | ±0.8 dB ±1.3 dB, 4.2GHz < f ≤ 6GHz |  |
| 6.6.2.3A.2_1 Adjacent Channel Leakage power Ratio for CA (inter-band DL CA and UL CA) for UL 64QAM | Same as 6.6.2.3A.2 |  |
| 6.6.2.3A.2_2 Adjacent Channel Leakage power Ratio for CA (inter-band DL CA and UL CA) for UL 256QAM | Same as 6.6.2.3A.2 |  |
| 6.6.2.3A.3 Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.6.2.3 |  |
| 6.6.2.3A.3_1 Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous DL CA and UL CA) for UL 64QAM | Same as 6.6.2.3 |  |
| 6.6.2.3A.3_2 Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous DL CA and UL CA) for UL 256QAM | Same as 6.6.2.3 |  |
| 6.6.2.3A.4.1 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous 3DL CA and 3UL CA) | Same as 6.6.2.3 |  |
| 6.6.2.3A.4.2 Adjacent Channel Leakage power Ratio for CA (inter-band 3DL CA and 3UL CA) | Same as 6.6.2.3 |  |
| 6.6.2.3A.4.3 Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous 3DL CA and 3UL CA) | Same as 6.6.2.3 |  |
| 6.6.2.3A.5 Adjacent Channel Leakage power Ratio for CA (4UL CA) | Same as 6.6.2.3 |  |
| 6.6.2.3B Adjacent Channel Leakage power Ratio for UL-MIMO | Same as 6.6.2.3 for each antenna |  |
| 6.6.2.3E Adjacent Channel Leakage power Ratio for UE category 0 | Same as 6.6.2.3 | Same as 6.6.2.3 |
| 6.6.2.3EA Adjacent Channel Leakage power Ratio for UE category M1 | Same as 6.6.2.3 |  |
| 6.6.2.3EB Adjacent Channel Leakage power Ratio for UE category 1bis | Same as 6.6.2.3 |  |
| 6.6.2.3_3 Adjacent Channel Leakage power Ratio for UL 64QAM | Same as 6.6.2.3 |  |
| 6.6.2.3_4 Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH with UL 64QAM | Same as 6.6.2.3 |  |
| 6.6.2.3F Adjacent Channel Leakage power Ratio for UE category NB1 | ±0.8 dB |  |
| 6.6.2.3G.1 Adjacent Channel Leakage power Ratio for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | ±0.8 dB |  |
| 6.6.2.3G. 2 Adjacent Channel Leakage power Ratio for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | Same as 6.6.2.3G.1 |  |
| 6.6.2.3G.3 Adjacent Channel Leakage power Ratio for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 6.6.2.3G.1 |  |
| 6.6.2.4 Void | Void |  |
| 6.6.3.1 Transmitter Spurious emissions | 9kHz < f ≤ 4 GHz: ± 2.0 dB 4 GHz < f ≤ 19 GHz: ± 4.0 dB |  |
| 6.6.3.1_1 Transmitter Spurious emissions for Multi-Cluster PUSCH | Same as 6.6.3.1 |  |
| 6.6.3.1A.1 Transmitter Spurious emissions for CA (intra-band contiguous DL CA and UL CA) | Same as 6.6.3.1 |  |
| 6.6.3.1A.2 Transmitter Spurious emissions for CA (inter-band DL CA and UL CA) | Same as 6.6.3.1 |  |
| 6.6.3.1A.3 Transmitter Spurious emissions for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.6.3.1 |  |
| 6.6.3.1A.4.1 Transmitter Spurious emissions for CA (3UL CA) (intra-band contiguous 3DL CA and 3UL CA) | Same as 6.6.3.1 |  |
| 6.6.3.1A.4.2 Transmitter Spurious emissions for CA (3UL CA) (inter-band 3DL CA and 3UL CA) | Same as 6.6.3.1 |  |
| 6.6.3.1A.4.3 Transmitter Spurious emissions for CA (3UL CA) (intra-band non-contiguous 3DL CA and 3UL CA) | Same as 6.6.3.1 |  |
| 6.6.3.1A.5 Transmitter Spurious emissions for CA (4UL CA) | Same as 6.6.3.1 |  |
| 6.6.3B.1Transmitter Spurious emissions for UL-MIMO | Same as 6.6.3.1, at each antenna used for transmission | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |
| 6.6.3E.1 Transmitter Spurious emissions for UE category 0 | Same as 6.6.3.1 |  |
| 6.6.3EA.1 Transmitter Spurious emissions for UE category M1 | Same as 6.6.3.1 |  |
| 6.6.3F.1 Transmitter Spurious emissions for category NB1 and NB2 | Same as 6.6.3.1 |  |
| 6.6.3G.1 Transmitter Spurious emissions for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | 9kHz < f ≤ 4 GHz: ± 2.0 dB 4 GHz < f ≤ 19 GHz: ± 4.0 dB 19GHz < f ≤ 26GHz: ± 6.0 dB |  |
| 6.6.3G.1_1 Spurious emission band UE co-existence for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Same as 6.6.3G.1 |  |
| 6.6.3G.2 Spurious emission for V2X Communication / Intra-band contiguous MCC operation | Same as 6.6.3G.1 |  |
| 6.6.3G.2_1 Spurious emission band UE co-existence for V2X Communication / Sidelink simultaneous with E-UTRA uplink transmissions | Same as 6.6.3G.1 |  |
| 6.6.3G.3 Spurious emission for V2X Communication / Intra-band contiguous MCC operation | Same as 6.6.3G.1 |  |
| 6.6.3G.3_1 Spurious emission band UE co-existence for V2X Communication / Intra-band contiguous MCC operation | Same as 6.6.3G.1 |  |
| 6.6.3.2 Spurious emission band UE co-existence | ± 2.0 dB for results > -60 dBm, f ≤ 3.0GHz ±2.5 dB, 3.0GHz < f ≤ 4.2GHz ± 3.0 dB for results ≤ -60 dBm, f ≤ 3.0GHz ±3.6 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.6.3.2A.1 Spurious emission band UE co-existence for CA (intra-band contiguous DL CA and UL CA) | Same as 6.6.3.2 |  |
| 6.6.3.2A.2 Spurious emission band UE co-existence for CA (inter-band DL CA and UL CA) | Same as 6.6.3.2 |  |
| 6.6.3.2A.3 Spurious emission band UE co-existence for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.6.3.2 |  |
| 6.6.3.2A.4 Spurious emission band UE co-existence for CA (3UL CA) | Same as 6.6.3.2 |  |
| 6.6.3.2A.5 Spurious emission band UE co-existence for CA (4UL CA) | Same as 6.6.3.2 |  |
| 6.6.3.3 Additional spurious emissions | 9kHz < f ≤ 4 GHz: ± 2.0 dB NS-07 769 ≤ f ≤ 775 MHz: ± 1.5 dB |  |
| 6.6.3.3_1 Additional spurious emissions for UL 64QAM | Same as 6.6.3.3 |  |
| 6.6.3.3_2 Additional spurious emissions for UL 256QAM | Same as 6.6.3.3 |  |
| 6.6.3.3A.1 Additional spurious emissions for CA (intra-band contiguous DL CA and UL CA) | 9kHz < f ≤ 4 GHz: ± 2.0 dB |  |
| 6.6.3.3A.1_1 Additional spurious emissions for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM | Same as 6.6.3.3A.1 |  |
| 6.6.3.3A.1_2 Additional spurious emissions for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM | Same as 6.6.3.3A.1 |  |
| 6.6.3.3A.2 Additional spurious emissions for CA (inter-band DL CA and UL CA) | Same as 6.6.3.3 |  |
| 6.6.3.3A.2_1 Additional spurious emissions for CA (inter-band DL CA and UL CA) for UL 64QAM | Same as 6.6.3.3 |  |
| 6.6.3.3A.2_2 Additional spurious emissions for CA (inter-band DL CA and UL CA) for UL 256QAM | Same as 6.6.3.3 |  |
| 6.6.3.3A.4 Additional spurious emissions for CA (3UL CA) | Same as 6.6.3.3A.1 |  |
| 6.6.3.3A.5 Additional spurious emissions for CA (4UL CA) | Same as 6.6.3.3A.1 |  |
| 6.6.3B.2 Spurious emission band UE co-existence for UL-MIMO | Same as 6.6.3.2 at each antenna used for transmission |  |
| 6.6.3B.3 Additional spurious emissions for UL-MIMO | Same as 6.6.3.3, at each antenna used for transmission |  |
| 6.6.3E.2 Spurious emission band UE co-existence for UE category 0 | Same as 6.6.3.2 |  |
| 6.6.3E.3 Additional spurious emissions for UE category 0 | 6.6.3.3 |  |
| 6.6.3EA.2 Spurious emission band UE co-existence for UE category M1 | Same as 6.6.3.2 |  |
| 6.6.3EA.3 Additional spurious emissions for UE category M1 | Same as 6.6.3.3 |  |
| 6.6.3EB.1 Transmitter Spurious emissions for UE category 1bis | Same as 6.6.3.2 |  |
| 6.6.3EB.2 Spurious emission band UE co-existence for UE category 1bis | Same as 6.6.3.2 |  |
| 6.6.3EB.3 Additional spurious emissions for UE category 1bis | Same as 6.6.3.3 |  |
| 6.6.3EC.1 Transmitter Spurious emissions for UE category M2 | Same as 6.6.3.1 |  |
| 6.6.3EC.2 Spurious emission band UE co-existence for UE category M2 | Same as 6.6.3.2 |  |
| 6.6.3EC.3 Additional spurious emissions for UE category M2 | Same as 6.6.3.3 |  |
| 6.6.3F.2 Spurious emission band UE co-existence for category NB1 UE | ± 2.0 dB for results > -60 dBm, f ≤ 3.0GHz ± 3.0 dB for results ≤ -60 dBm, f ≤ 3.0GHz |  |
| 6.7 Transmit intermodulation | ± 2.6 dB, f ≤ 3.0GHz ±3.6 dB, 3.0GHz < f ≤ 4.2GHz | Overall system uncertainty comprises four quantities: 1. Wanted signal setting error 2. CW Interferer level error 3. Wanted signal meas. error 4. Intermodulation product measurement error The relative level of the wanted signal and the CW interferer has 2 x effect on the intermodulation product. Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared to provide the combined effect. Test System uncertainty = SQRT [(2 x SQRT (Wanted setting_error2 + CW_level_error2)) 2 + Wanted_level_meas error2 + Intermodulation product measurement error2] f ≤ 3.0GHz Wanted signal setting ± 0.7dB CW Interferer level ± 1.0dB Wanted signal meas ± 0.7dB Intermodulation product measurement error ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal setting ± 1.0dB CW Interferer level ± 1.3dB Wanted signal meas ± 1.0dB Intermodulation product measurement error ± 1.0dB |
| 6.7A.1 Transmit intermodulation for CA (intra-band contiguous DL CA and UL CA) | Same as 6.7, at each antenna used for transmission |  |
| 6.7A.2 Transmit intermodulation for CA (inter-band DL CA and UL CA) | Same as 6.7, at each antenna used for transmission |  |
| 6.7A.2 Transmit intermodulation for CA (4UL CA) | Same as 6.7, at each antenna used for transmission |  |
| 6.7E Transmit intermodulation for UE category 0 | Same as 6.7 |  |
| 6.7EA Transmit intermodulation for UE category M1 | Same as 6.7 |  |
| 6.7EB Transmit intermodulation for UE category 1bis | Same as 6.7 |  |
| 6.7EC Transmit intermodulation for UE category M2 | Same as 6.7 |  |
| 6.7F Transmit intermodulation for UE category NB1 and NB2 | Same as 6.7 |  |
| 6.7G.1 Transmit intermodulation for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Same as 6.7 |  |
| 6.7G.2 Transmit intermodulation for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | Same as 6.7 |  |
| 6.7G.3 Transmit intermodulation for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 6.7 |  |
| 6.8B Time alignment error for UL-MIMO | ±25 ns |  |

## F.1.3	Measurement of receiver

Table F.1.3-1: Maximum Test System Uncertainty for receiver tests

| Subclause |  | Maximum Test System Uncertainty1 |  | Derivation of Test System Uncertainty |  |
| --- | --- | --- | --- | --- | --- |
| 7.3 Reference sensitivity power level |  | Downlink power ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |  |  |
| 7.3_1 Reference sensitivity level with 4 Rx antenna ports |  | Same as 7.3 |  |  |  |
| 7.3A.1 Reference sensitivity level for CA (intra-band contiguous DL CA and UL CA) |  | Same as 7.3 for each CC |  |  |  |
| 7.3A.2 Reference sensitivity level for CA (intra-band contiguous DL CA without UL CA) |  | Same as 7.3A.1 |  |  |  |
| 7.3A.3 Reference sensitivity level for CA (inter-band DL CA without UL CA) |  | Downlink power ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 6GHz |  |  |  |
| 7.3A.4 Reference sensitivity level for CA (intra-band non-contiguous DL CA without UL CA) |  | Same as 7.3A.1 |  |  |  |
| 7.3A.5 Reference sensitivity level for 3DL CA |  | Same as 7.3A.3 |  |  |  |
| 7.3A.6 Reference sensitivity level for CA (inter-band DL CA and UL CA) |  | Same as 7.3A.1 |  |  |  |
| 7.3A.7 Reference sensitivity level for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 7.3A.1 |  |  |  |
| 7.3A.9 Reference sensitivity level for 4DL CA |  | Same as 7.3A.3 |  |  |  |
| 7.3A.10 Reference sensitivity level for 5DL CA |  | Same as 7.3A.3 |  |  |  |
| 7.3A.11 Reference sensitivity level for 6DL CA |  | Same as 7.3A.1 |  |  |  |
| 7.3B Reference Sensitivity Level for UL-MIMO |  | Same as 7.3 |  |  |  |
| 7.3D.1 Reference sensitivity level for ProSe Direct Discovery |  | Sidelink power ±0.7 dB, f ≤ 3.0GHz |  |  |  |
| 7.3D.2 Reference sensitivity level for ProSe Direct Communication |  | Same as 7.3D.1 |  |  |  |
| 7.3E Reference Sensitivity Level for UE category 0 |  | Same as 7.3 |  |  |  |
| 7.3EA Reference Sensitivity Level for UE category M1 |  | Same as 7.3 |  |  |  |
| 7.3EB Reference sensitivity level for UE category 1bis |  | Same as 7.3 |  |  |  |
| 7.3EC Reference Sensitivity Level for UE category M2 |  | Same as 7.3 |  |  |  |
| 7.3F.1 Reference sensitivity level without repetitions for UE category NB1 and NB2 |  | Downlink power ±0.7 dB, f ≤ 3.0GHz |  |  |  |
| 7.3F.2 Reference sensitivity level with repetitions for category NB1 |  | Downlink power ±0.7 dB, f ≤ 3.0GHz |  |  |  |
| 7.3G.1 Reference sensitivity level for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Downlink power ±1.5 dB, 4.2GHz < f ≤ 6GHz |  |  |  |
| 7.3G.2 Reference sensitivity level for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Downlink power ±1.5 dB, 4.2GHz < f ≤ 6GHz for V2X sidelink Downlink power ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz for E-UTRA |  |  |  |
| 7.3G.3 Reference sensitivity level for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 7.3G.1 |  |  |  |
| 7.4 Maximum input level |  | Downlink power ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |  |  |
| 7.4_1 Maximum input level with 4 Rx antenna ports |  | Same as 7.4 |  |  |  |
| 7.4A.1 Maximum input level for CA (intra band contiguous DL CA and UL CA) |  | Same as 7.4 for each CC |  | Uncertainties apply for each CC |  |
| 7.4A.1 H Maximum input level for SCE-L1 for CA (intra-band contiguous DL CA and UL CA) |  | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.2 Maximum input level for CA (intra band contiguous DL CA without UL CA) |  | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.2 H Maximum input level for SCE-L1 for CA (intra-band contiguous DL CA without UL CA) |  | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.3 Maximum input level for CA (inter-band DL CA without UL CA) |  | Downlink power ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 6GHz Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.3_H Maximum input level for CA (inter-band DL CA without UL CA) for 256QAM in DL |  | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.4 Maximum input level for CA (intra band non-contiguous DL CA without UL CA) |  | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.5 Maximum input level for 3DL CA |  | Downlink power same as 7.4A.3 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.5_H Maximum input level for SCE-L1 for 3DL CA |  | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.7 Maximum input level for 4DL CA |  | Downlink power same as 7.4A.3 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.7_H Maximum input level for 4DL CA for 256QAM in DL |  | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.8 Maximum input level for 5DL CA |  | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.8_H Maximum input level for 5DL CA for 256QAM in DL |  | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.9 Maximum input level for 6DL CA |  | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.9_H Maximum input level for 6DL CA for 256QAM in DL |  | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4B Maximum Input Level for UL-MIMO |  | Downlink power same as 7.4 Uplink power measurement same as 7.4, at each antenna connector used for transmission |  | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |  |
| 7.4D.1 Maximum input level for ProSe Direct Discovery |  | Sidelink power ±0.7 dB, f ≤ 3.0GHz |  |  |  |
| 7.4D.2 Maximum input level for ProSe Direct Communication |  | Same as 7.4D.1 |  |  |  |
| 7.4E Maximum input level for UE category 0 |  | Same as 7.4 |  |  |  |
| 7.4EA Maximum input level for UE category M1 |  | Same as 7.4 |  |  |  |
| 7.4EB Maximum input level for UE Category 1bis |  | Same as 7.4 |  |  |  |
| 7.4EC Maximum input level for UE category M2 |  | Same as 7.4 |  |  |  |
| 7.4F Maximum input level for UE category NB1 |  | Downlink power ±0.7 dB, f ≤ 3.0GHz |  |  |  |
| 7.4G.1 Maximum input level for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Downlink power ±1.5 dB, 4.2GHz < f ≤ 6GHz |  |  |  |
| 7.4G.2 Maximum input level for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Downlink power ±1.5 dB, 4.2GHz < f ≤ 6GHz for V2X sidelink Downlink power ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz for E-UTRA Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz for E-UTRA |  |  |  |
| 7.4G.3 Maximum input level for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 7.4G.1 |  |  |  |
| 7.4_H Maximum input level for SCE-L1 |  | Same as 7.4 |  |  |  |
| 7.5 Adjacent Channel Selectivity (ACS) |  | ACS value ±1.1 dB, f ≤ 3.0GHz ±1.5 dB, 3.0GHz < f ≤ 4.2GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  | Overall ACS uncertainty comprises three quantities: 1. Wanted signal level error 2. Interferer signal level error 3. Additional impact of interferer ACLR Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The interferer ACLR effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect. f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB Interferer signal level ± 1.0dB f ≤ 4.2GHz Impact of interferer ACLR 0.1dB |  |
| 7.5_1 Adjacent Channel Selectivity (ACS) with 4 Rx antenna ports |  | Same as 7.5 |  |  |  |
| 7.5A.1 Adjacent Channel Selectivity (ACS) for CA (intra band contiguous DL CA and UL CA) |  | Same as 7.5 for each CC |  | Same as 7.5 The wanted signal level uncertainty applies for each CC. Overall ACS uncertainty calculation includes the uncertainty for wanted level error only once, as the uncertainty of other CCs is not expected to have any significant effect. |  |
| 7.5A.2 Adjacent Channel Selectivity (ACS) for CA (intra band contiguous DL CA without UL CA) |  | Downlink power same as 7.5 Uplink power measurement same as 7.5 |  | The wanted signal level uncertainty applies for each CC |  |
| 7.5A.3 Adjacent Channel Selectivity (ACS) for CA (inter band DL CA without UL CA) |  | ACS value ±1.1 dB, f ≤ 3.0GHz ±1.5 dB, 3.0GHz < f ≤ 4.2GHz ±2.2 dB, 4.2GHz < f ≤ 6GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  | The wanted signal level uncertainty applies for each CC Overall ACS uncertainty comprises three quantities: 1. Wanted signal level error 2. Interferer signal level error 3. Additional impact of interferer ACLR Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The interferer ACLR effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect. f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB Interferer signal level ± 1.0dB 4.2GHz < f ≤ 6GHz Wanted signal level ± 1.5dB Interferer signal level ± 1.5dB f ≤ 6GHz Impact of interferer ACLR 0.1dB |  |
| 7.5A.4 Adjacent Channel Selectivity (ACS) for CA (intra-band non-contiguous DL CA without UL CA) |  | Downlink power same as 7.5 Uplink power measurement same as 7.5 |  | The wanted signal level uncertainty applies for each CC |  |
| 7.5A.5 Adjacent Channel Selectivity (ACS) for 3DL CA |  | Downlink power same as 7.5A.3 Uplink power measurement same as 7.5 |  | The wanted signal level uncertainty applies for each CC |  |
| 7.5A.7 Adjacent Channel Selectivity (ACS) for 4DL CA |  | Downlink power same as 7.5A.3 Uplink power measurement same as 7.5 |  | The wanted signal level uncertainty applies for each CC |  |
| 7.5A.8 Adjacent Channel Selectivity (ACS) for 5DL CA |  | Downlink power same as 7.5A.3 Uplink power measurement same as 7.5 |  | The wanted signal level uncertainty applies for each CC |  |
| 7.5A.9 Adjacent Channel Selectivity (ACS) for 6DL CA |  | Downlink power same as 7.5A.3 Uplink power measurement same as 7.5 |  | The wanted signal level uncertainty applies for each CC |  |
| 7.5B Adjacent Channel Selectivity (ACS) for UL-MIMO |  | ACS value same as 7.5 Uplink power measurement same as 7.5, at each antenna connector used for transmission |  | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |  |
| 7.5D.1 Adjacent channel selectivity (ACS) for ProSe Direct Discovery |  | ACS value ±1.1 dB, f ≤ 3.0GHz |  | Overall ACS uncertainty comprises three quantities: 1. Wanted signal level error 2. Interferer signal level error 3. Additional impact of interferer ACLR Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The interferer ACLR effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect. f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level ± 0.7dB f ≤ 4.2GHz Impact of interferer ACLR 0.1dB |  |
| 7.5D.2 Adjacent channel selectivity (ACS) for ProSe Direct Communication |  | Same as 7.5D.1 |  | Same as 7.5D.1 |  |
| 7.5E Adjacent Channel Selectivity (ACS) for UE category 0 |  | Same as 7.5 |  | Same as 7.5 |  |
| 7.5EA Adjacent Channel Selectivity (ACS) for UE category M1 |  | Same as 7.5 |  | Same as 7.5 |  |
| 7.5EB Adjacent Channel Selectivity (ACS) for UE Category 1bis |  | Same as 7.5 |  | Same as 7.5 |  |
| 7.5EC Adjacent Channel Selectivity (ACS) for UE category M2 |  | Same as 7.5 |  | Same as 7.5 |  |
| 7.5F Adjacent Channel Selectivity (ACS) for UE category NB1 |  | ACS value ±1.1 dB, f ≤ 3.0GHz |  | Overall ACS uncertainty comprises three quantities: 1. Wanted signal level error 2. Interferer signal level error 3. Additional impact of interferer ACLR Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The interferer ACLR effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect. f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level ± 0.7dB Impact of interferer ACLR 0.1dB |  |
| 7.5G.1 Adjacent channel selectivity (ACS) for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | ACS ±2.2 dB, 4.2GHz < f ≤ 6GHz |  | Overall ACS uncertainty comprises three quantities: 1. Wanted signal level error 2. Interferer signal level error 3. Additional impact of interferer ACLR Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The interferer ACLR effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect. 4.2GHz < f ≤ 6GHz Wanted signal level ± 1.5dB Interferer signal level ± 1.5dB Impact of interferer ACLR 0.1dB |  |
| 7.5G.2 Adjacent channel selectivity (ACS) for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | ACS ±2.2 dB, 4.2GHz < f ≤ 6GHz for V2X sidelink ACS value ±1.1 dB, f ≤ 3.0GHz ±1.5 dB, 3.0GHz < f ≤ 4.2GHz for E-UTRA Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz for E-UTRA ±1.5 dB, 4.2GHz < f ≤ 6GHz for V2X sidelink |  | Overall ACS uncertainty comprises three quantities: 1. Wanted signal level error 2. Interferer signal level error 3. Additional impact of interferer ACLR Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The interferer ACLR effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect. f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB Interferer signal level ± 1.0dB f ≤ 4.2GHz Impact of interferer ACLR 0.1dB Interferer signal level ± 1.0dB 4.2GHz < f ≤ 6GHz Wanted signal level ± 1.5dB Interferer signal level ± 1.5dB Impact of interferer ACLR 0.1dB |  |
| 7.5G.3 Adjacent channel selectivity (ACS) for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 7.5G.1 |  | Same as 7.5G.1 |  |
| 7.6.1 In-band blocking |  | Blocking ±1.4 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  | Overall blocking uncertainty can have these contributions: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer ACLR 4. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer ACLR or Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect + Broadband noise effect. In-band blocking, using modulated interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB Interferer signal level ± 1.0dB f ≤ 4.2GHz Interferer ACLR 0.4dB Broadband noise not applicable |  |
| 7.6.1_1 In-band blocking with 4 Rx antenna ports |  | Same as 7.6.1 |  | Same as 7.6.1 |  |
| 7.6.1A.1 In-band blocking for CA (intra band contiguous DL CA and UL CA) |  | Same as 7.6.1 for each CC |  | Same as 7.6.1 The wanted signal level uncertainty applies for each CC. Overall blocking uncertainty calculation includes the uncertainty for wanted level error only once, as the uncertainty of other CCs is not expected to have any significant effect. |  |
| 7.6.1A.2 In-band blocking for CA (intra band contiguous DL CA without UL CA) |  | Same as 7.6.1A.1 |  | Same as 7.6.1A.1 |  |
| 7.6.1A.3 In-band blocking for CA (inter band DL CA without UL CA) |  | Blocking ±1.4 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz ±2.5 dB, 4.2GHz < f ≤ 6.0GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  | Overall blocking uncertainty can have these contributions: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer ACLR 4. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer ACLR or Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect + Broadband noise effect. In-band blocking, using modulated interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB Interferer signal level ± 1.0dB 4.2GHz < f ≤ 6.0GHz Wanted signal level ± 1.5dB Interferer signal level ± 1.5dB f ≤ 6.0GHz Interferer ACLR 0.4dB Broadband noise not applicable |  |
| 7.6.1A.4 In-band blocking for CA (intra-band non-contiguous DL CA without UL CA) |  | Same as 7.6.1A.1 |  | Same as 7.6.1A.1 |  |
| 7.6.1A.5 In-band blocking for 3DL CA |  | Same as 7.6.1A.3 |  | Same as 7.6.1A.1 |  |
| 7.6.1A.7 In-band blocking for 4DL CA |  | Same as 7.6.1A.3 |  | Same as 7.6.1A.1 |  |
| 7.6.1A.8 In-band blocking for 5DL CA |  | Same as 7.6.1A.3 |  | Same as 7.6.1A.1 |  |
| 7.6.1A.9 In-band blocking for 6DL CA |  | Same as 7.6.1A.3 |  | Same as 7.6.1A.1 |  |
| 7.6.1A.10 In-band blocking for 7DL CA |  | Same as 7.6.1A.3 |  | Same as 7.6.1A.1 |  |
| 7.6.1B In-band blocking for UL-MIMO |  | Blocking same as 7.6.1 Uplink power measurement same as 7.6.1, at each antenna connector used for transmission |  | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |  |
| 7.6.1D.1 In-band blocking for ProSe Direct Discovery |  | Blocking ±1.4 dB, f ≤ 3.0GHz |  | Overall blocking uncertainty can have these contributions: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer ACLR 4. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer ACLR or Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect + Broadband noise effect. In-band blocking, using modulated interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB f ≤ 4.2GHz Interferer ACLR 0.4dB Broadband noise not applicable |  |
| 7.6.1D.2 In-band blocking for ProSe Direct Communication |  | Same as 7.6.1D.1 |  | Same as 7.6.1D.1 |  |
| 7.6.1E In-band blocking for UE category 0 |  | Same as 7.6.1 |  | Same as 7.6.1 |  |
| 7.6.1EB In-band blocking for UE Category 1bis |  | Same as 7.6.1 |  | Same as 7.6.1 |  |
| 7.6.1F In-band blocking for category NB1 and NB2 |  | Blocking ±1.4 dB, f ≤ 3.0GHz |  | Overall blocking uncertainty can have these contributions: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer ACLR 4. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer ACLR or Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect + Broadband noise effect. In-band blocking, using modulated interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB |  |
| 7.6.1G.1 In-band blocking for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Blocking ±2.5 dB, 4.2GHz < f ≤ 6.0GHz |  | Overall blocking uncertainty can have these contributions: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer ACLR 4. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer ACLR or Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect + Broadband noise effect. In-band blocking, using modulated interferer: 4.2GHz < f ≤ 6.0GHz Wanted signal level ± 1.5dB Interferer signal level: ± 1.5dB 4.2GHz < f ≤ 6.0GHz Interferer ACLR 0.4dB Broadband noise not applicable |  |
| 7.6.1G.3 In-band blocking for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 7.6.1G.1 |  | Same as 7.6.1G.1 |  |
| 7.6.1G.2 In-band blocking for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Blocking ±2.5 dB, 4.2GHz < f ≤ 6.0GHz for V2X sidelink Blocking ±1.4 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz for E-UTRA Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz for E-UTRA ±1.5 dB, 4.2GHz < f ≤ 6GHz for V2X sidelink |  | Overall blocking uncertainty can have these contributions: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer ACLR 4. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer ACLR or Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect + Broadband noise effect. In-band blocking, using modulated interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB Interferer signal level ± 1.0dB 4.2GHz < f ≤ 6.0GHz Wanted signal level ± 1.5dB Interferer signal level: ± 1.5dB f ≤ 4.2GHz, 4.2GHz < f ≤ 6.0GHz Interferer ACLR 0.4dB Broadband noise not applicable |  |
| 7.6.1G.3 In-band blocking for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 7.6.1G.1 |  | Same as 7.6.1G.1 |  |
| 7.6.1EA In-band blocking for UE category M1 |  | Blocking ±1.4 dB, f ≤ 3.0GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz |  | Overall blocking uncertainty can have these contributions: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer ACLR 4. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer ACLR or Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect + Broadband noise effect. In-band blocking, using modulated interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB |  |
| 7.6.1EC In-band blocking for UE category M2 |  | Same as 7.6.1EA |  | Same as 7.6.1EA |  |
| 7.6.2 Out of-band blocking |  | Wanted signal f ≤ 3.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.3 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.2 dB Uplink power measurement ±0.7 dB Wanted signal 3.0GHz < f ≤ 4.2GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.5 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.3 dB Uplink power measurement ±1.0 dB |  | Out of band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB Interferer signal level: ± 1.0dB up to 3GHz ± 3.0dB up to 12.75GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.2_1 Out of-band blocking with 4 Rx antenna ports |  | Same as 7.6.2 |  | Same as 7.6.2 |  |
| 7.6.2A.1 Out-of-band blocking for CA (intra band contiguous DL CA and UL CA) |  | Same as 7.6.2 for each CC |  | Same as 7.6.2 The wanted signal level uncertainty applies for each CC. Overall blocking uncertainty calculation includes the uncertainty for wanted level error only once, as the uncertainty of other CCs is not expected to have any significant effect. |  |
| 7.6.2A.2 Out-of-band blocking for CA (intra band contiguous DL CA without UL CA) |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.6.2A.3 Out-of-band blocking for CA (inter band DL CA without UL CA) |  | Wanted signal f ≤ 3.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.3 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.2 dB Uplink power measurement ±0.7 dB Wanted signal 3.0GHz < f ≤ 4.2GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.5 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.3 dB Uplink power measurement ±1.0 dB Wanted signal 4.2GHz < f ≤ 6.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.9 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.5 dB |  | Out of band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB 4.2GHz < f ≤ 6.0GHz Wanted signal level ± 1.5dB Interferer signal level: ± 1.0dB up to 3GHz ± 3.0dB up to 12.75GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.2A.4 Out-of-band blocking for CA (intra-band non-contiguous DL CA without UL CA) |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.6.2A.5 Out-of-band blocking for 3DL CA |  | Same as 7.6.2A.3 |  | Same as 7.6.2A.1 |  |
| 7.6.2A.7 Out-of-band blocking for 4DL CA |  | Same as 7.6.2A.3 |  | Same as 7.6.2A.1 |  |
| 7.6.2A.8 Out-of-band blocking for 5DL CA |  | Same as 7.6.2A.3 |  | Same as 7.6.2A.1 |  |
| 7.6.2A.9 Out-of-band blocking for 6DL CA |  | Same as 7.6.2A.3 |  | Same as 7.6.2A.1 |  |
| 7.6.2A.10 Out-of-band blocking for 7DL CA |  | Same as 7.6.2A.3 |  | Same as 7.6.2A.1 |  |
| 7.6.2B Out-of-band blocking for UL-MIMO |  | Blocking same as 7.6.2 Uplink power measurement same as 7.6.2, at each antenna connector used for transmission |  | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |  |
| 7.6.2D.1 Out-of-band blocking for ProSe Direct Discovery |  | Wanted signal f ≤ 3.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.3 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.2 dB ±0.7 dB |  | Out of band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 1.0dB up to 3GHz ± 3.0dB up to 12.75GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.2D.2 Out-of-band blocking for ProSe Direct Communication |  | Same as 7.6.2D.1 |  | Same as 7.6.2D.1 |  |
| 7.6.2E Out of-band blocking for UE category 0 |  | Same as 7.6.2 |  | Same as 7.6.2 |  |
| 7.6.2F Out of-band blocking for category NB1 and NB2 |  | Wanted signal f ≤ 3.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.3 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.2 dB |  | Out of band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 1.0dB up to 3GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.2G.1 Out-of-band blocking for V2X Communication/ Non-concurrent with E-UTRA uplink transmissions |  | Wanted signal 4.2GHz < f ≤ 6.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.9 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.5 dB |  | Out of band blocking, using CW interferer: 4.2GHz < f ≤ 6.0GHz Wanted signal level ± 1.5dB Interferer signal level: ± 1.0dB up to 3GHz ± 3.0dB up to 12.75GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.2G.2 Out-of-band blocking for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Wanted signal 4.2GHz < f ≤ 6.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.9 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.5 dB |  | Out of band blocking, using CW interferer: 4.2GHz < f ≤ 6.0GHz Wanted signal level ± 1.5dB Interferer signal level: ± 1.0dB up to 3GHz ± 3.0dB up to 12.75GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.2G.3 Out-of-band blocking for V2X Communication / Intra-band contiguous multi-carrier operation |  | Wanted signal 4.2GHz < f ≤ 6.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.9 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.5 dB |  | Out of band blocking, using CW interferer: 4.2GHz < f ≤ 6.0GHz Wanted signal level ± 1.5dB Interferer signal level: ± 1.0dB up to 3GHz ± 3.0dB up to 12.75GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.2EA Out-of-band blocking for UE category M1 |  | Wanted signal f ≤ 3.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.3 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.2 dB Uplink power measurement ±0.7 dB |  | Out of band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 1.0dB up to 3GHz ± 3.0dB up to 12.75GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.2EB Out-of-band blocking for UE category 1bis |  | Same as 7.6.2 |  | Same as 7.6.2 |  |
| 7.6.2EC Out-of-band blocking for UE category M2 |  | Same as 7.6.2 |  | Same as 7.6.2 |  |
| 7.6.3 Narrow band blocking |  | Blocking ±1. 1 dB, f ≤ 3.0GHz ±1. 5 dB, 3.0GHz < f ≤ 4.2GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  | Narrow band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB Interferer signal level ± 1.0dB Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.3_1 Narrow band blocking with 4 Rx antenna ports |  | Same as 7.6.3 |  | Same as 7.6.3 |  |
| 7.6.3A.1 Narrow band blocking for CA (intra band contiguous DL CA and UL CA) |  | Same as 7.6.3 for each CC |  | Same as 7.6.3 The wanted signal level uncertainty applies for each CC. Overall blocking uncertainty calculation includes the uncertainty for wanted level error only once, as the uncertainty of other CCs is not expected to have any significant effect. |  |
| 7.6.3A.2 Narrow band blocking for CA (intra band contiguous DL CA without UL CA) |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.3 Narrow band blocking for CA (inter band DL CA without UL CA) |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.4 Narrow band blocking for CA (intra-band non-contiguous DL CA without UL CA) |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.5 Narrow band blocking for 3DL CA |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.7 Narrow band blocking for 4DL CA |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.8 Narrow band blocking for 5DL CA |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.9 Narrow band blocking for 6DL CA |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.10 Narrow band blocking for 7DL CA |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3B Narrow band blocking for UL-MIMO |  | Blocking same as 7.6.3 Uplink power measurement same as 7.6.3, at each antenna connector used for transmission |  | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |  |
| 7.6.3D.1 Narrow band blocking for ProSe Direct Discovery |  | Blocking ±1.1 dB, f ≤ 3.0GHz |  | Narrow band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.3D.2 Narrow band blocking for ProSe Direct Communication |  | Same as 7.6.3D.1 |  | Same as 7.6.3D.1 |  |
| 7.6.3E Narrow band blocking for UE category 0 |  | Same as 7.6.3 |  | Same as 7.6.3 |  |
| 7.6.3EA Narrow band blocking for UE category M1 |  | Blocking ±1. 1 dB, f ≤ 3.0GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz |  | Narrow band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.3EB Narrow band blocking for UE category 1bis |  | Same as 7.6.3 |  | Same as 7.6.3 |  |
| 7.6.3EC Narrow band blocking for UE category M2 |  | Same as 7.6.3EA |  | Same as 7.6.3EA |  |
| 7.7 Spurious response |  | Same as 7.6.2 |  | Same as 7.6.2. |  |
| 7.7_1 Spurious response with 4 Rx antenna ports |  | Same as 7.6.2 |  | Same as 7.6.2 |  |
| 7.7A.1 Spurious response for CA (intra band contiguous DL CA and UL CA) |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.7A.2 Spurious response for CA (intra band contiguous DL CA without UL CA) |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.7A.3 Spurious response for CA (inter band DL CA without UL CA) |  | Same as 7.6.2A.3 |  | Same as 7.6.2A.1 |  |
| 7.7A.4 Spurious response for CA (intra-band non-contiguous DL CA without UL CA) |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.7A.5 Spurious response for 3DL CA |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.7A.7 Spurious response for 4DL CA |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.7A.8 Spurious response for 5DL CA |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.7A.9 Spurious response for 6DL CA |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.7B Spurious response for UL-MIMO |  | Same as 7.7 Uplink power measurement same as 7.6.2, at each antenna connector used for transmission |  | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |  |
| 7.7D.1 Spurious response for ProSe Direct Discovery |  | Same as 7.6.2D.1 |  | Same as 7.6.2D.1 |  |
| 7.7D.2 Spurious response for ProSe Direct Communication |  | Same as 7.6.2D.1 |  | Same as 7.6.2D.1 |  |
| 7.7E Spurious response for UE category 0 |  | Same as 7.7 |  | Same as 7.7 |  |
| 7.7EA Spurious response for UE category M1 |  | Same as 7.7 |  | Same as 7.7 |  |
| 7.7EB Spurious response for UE Category 1bis |  | Same as 7.7 |  | Same as 7.7 |  |
| 7.7EC Spurious response for UE category M2 |  | Same as 7.7 |  | Same as 7.7 |  |
| 7.7F Spurious response for category NB1 and NB2 |  | Wanted signal f ≤ 3.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.3 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.2 dB |  | Out of band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 1.0dB up to 3GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.7G.1 Spurious response for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Wanted signal 4.2GHz < f ≤ 6.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.9 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.5 dB |  | Out of band blocking, using CW interferer: 4.2GHz < f ≤ 6.0GHz Wanted signal level ± 1.5dB Interferer signal level: ± 1.0dB up to 3GHz ± 3.0dB up to 12.75GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.7G.2 Spurious response for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Wanted signal 4.2GHz < f ≤ 6.0GHz for V2X sidelink Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.9 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.5 dB Same as 7.6.2 for E-UTRA |  | Same as 7.7G.1 Same as 7.6.2 |  |
| 7.7G.3 Spurious response for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 7.7G.1 |  | Same as 7.7G.1 |  |
| 7.8.1 Wide band intermodulation |  | Intermodulation ±1.4 dB, f ≤ 3.0GHz ±2.6 dB, 3.0GHz < f ≤ 4.2GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  | Overall intermodulation uncertainty comprises three quantities: 1. Wanted signal level error 2. CW Interferer level error 3. Modulated Interferer level error Effect of interferer ACLR has not been included as modulated interferer has larger frequency offset The effect of the closer CW signal has twice the effect. Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared to provide the combined effect of the three signals. Test System uncertainty = SQRT [(2 x CW_level_error)2 +(mod interferer_level_error)2 +(wanted signal_level_error)2] f ≤ 3.0GHz Wanted signal level ± 0.7dB CW Interferer level ± 0.5dB Mod Interferer level ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB CW Interferer level ± 0.8dB Mod Interferer level ± 1.0dB |  |
| 7.8.1_1 Wide band Intermodulation with 4 Rx antenna ports |  | Same as 7.8.1 |  | Same as 7.8.1 |  |
| 7.8.1A.1 Wideband intermodulation for CA (intra band contiguous DL CA and UL CA) |  | Same as 7.8.1 for each CC |  | Same as 7.8.1 The wanted signal level uncertainty applies for each CC. Overall intermodulation uncertainty calculation includes the uncertainty for wanted level error only once, as the uncertainty of other CCs is not expected to have any significant effect. |  |
| 7.8.1A.2 Wideband intermodulation for CA (intra band contiguous DL CA without UL CA) |  | Same as 7.8.1A.1 |  | Same as 7.8.1A.1 |  |
| 7.8.1A.3 Wideband intermodulation for CA (inter band DL CA without UL CA) |  | Intermodulation ±1.4 dB, f ≤ 3.0GHz ±2.6 dB, 3.0GHz < f ≤ 4.2GHz ±3.8 dB, 4.2GHz < f ≤ 6GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  | Overall intermodulation uncertainty comprises three quantities: 1. Wanted signal level error 2. CW Interferer level error 3. Modulated Interferer level error Effect of interferer ACLR has not been included as modulated interferer has larger frequency offset The effect of the closer CW signal has twice the effect. Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared to provide the combined effect of the three signals. Test System uncertainty = SQRT [(2 x CW_level_error)2 +(mod interferer_level_error)2 +(wanted signal_level_error)2] f ≤ 3.0GHz Wanted signal level ± 0.7dB CW Interferer level ± 0.5dB Mod Interferer level ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB CW Interferer level ± 0.8dB Mod Interferer level ± 1.0dB 4.2GHz < f ≤ 6GHz Wanted signal level ± 1.5dB CW Interferer level ± 1.3dB Mod Interferer level ± 1.5dB |  |
| 7.8.1A.4 Wideband intermodulation for CA (intra band non-contiguous DL CA without UL CA) |  | Same as 7.8.1A.1 |  | Same as 7.8.1A.1 |  |
| 7.8.1A.5 Wideband intermodulation for 3DL CA |  | Same as 7.8.1A.3 |  | Same as 7.8.1A.1 |  |
| 7.8.1A.7 Wideband intermodulation for 4DL CA |  | Same as 7.8.1A.3 |  | Same as 7.8.1A.1 |  |
| 7.8.1A.8 Wideband intermodulation for 5DL CA |  | Same as 7.8.1A.3 |  | Same as 7.8.1A.1 |  |
| 7.8.1A.9 Wideband intermodulation for 6DL CA |  | Same as 7.8.1A.3 |  | Same as 7.8.1A.1 |  |
| 7.8.1B Wide band intermodulation for UL-MIMO |  | Intermodulation same as 7.8.1 Uplink power measurement same as 7.8.1, at each antenna connector used for transmission |  | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |  |
| 7.8.1D.1 Wide band Intermodulation for ProSe Direct Discovery |  | Intermodulation ±1.4 dB, f ≤ 3.0GHz ±2 |  | Overall intermodulation uncertainty comprises three quantities: 1. Wanted signal level error 2. CW Interferer level error 3. Modulated Interferer level error Effect of interferer ACLR has not been included as modulated interferer has larger frequency offset The effect of the closer CW signal has twice the effect. Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared to provide the combined effect of the three signals. Test System uncertainty = SQRT [(2 x CW_level_error)2 +(mod interferer_level_error)2 +(wanted signal_level_error)2] f ≤ 3.0GHz Wanted signal level ± 0.7dB CW Interferer level ± 0.5dB Mod Interferer level ± 0.7dB |  |
| 7.8.1D.2 Wide band Intermodulation for ProSe Direct Communication |  | Same as 7.8.1D.2 |  | Same as 7.8.1D.2 |  |
| 7.8.1E Wide band intermodulation for UE category 0 |  | Same as 7.8.1 |  | Same as 7.8.1 |  |
| 7.8.1EA Wide band Intermodulation for UE category M1 |  | Same as 7.8.1 |  | Same as 7.8.1 |  |
| 7.8.1EB Wide band Intermodulation for UE category 1bis |  | Same as 7.8.1 |  | Same as 7.8.1 |  |
| 7.8.1EC Wide band Intermodulation for UE category M2 |  | Same as 7.8.1 |  | Same as 7.8.1 |  |
| 7.8.1F Wide band intermodulation for category NB1 and NB2 |  | Intermodulation ±1.4 dB, f ≤ 3.0GHz |  | Overall intermodulation uncertainty comprises three quantities: 1. Wanted signal level error 2. CW Interferer level error 3. Modulated Interferer level error Effect of interferer ACLR has not been included as modulated interferer has larger frequency offset The effect of the closer CW signal has twice the effect. Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared to provide the combined effect of the three signals. Test System uncertainty = SQRT [(2 x CW_level_error)2 +(mod interferer_level_error)2 +(wanted signal_level_error)2] f ≤ 3.0GHz Wanted signal level ± 0.7dB CW Interferer level ± 0.5dB Mod Interferer level ± 0.7dB |  |
| 7.8.1G.1 Wide band Intermodulation for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Intermodulation ±3.8 dB, 4.2GHz < f ≤ 6GHz |  | Overall intermodulation uncertainty comprises three quantities: 1. Wanted signal level error 2. CW Interferer level error 3. Modulated Interferer level error Effect of interferer ACLR has not been included as modulated interferer has larger frequency offset The effect of the closer CW signal has twice the effect. Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared to provide the combined effect of the three signals. Test System uncertainty = SQRT [(2 x CW_level_error)2 +(mod interferer_level_error)2 +(wanted signal_level_error)2] 4.2GHz < f ≤ 6GHz Wanted signal level ± 1.5dB CW Interferer level ± 1.3dB Mod Interferer level ± 1.5dB |  |
| 7.8.1G.2 Wide band Intermodulation for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Intermodulation ±1.4 dB, f ≤ 3.0GHz ±2.6 dB, 3.0GHz < f ≤ 4.2GHz ±3.8 dB, 4.2GHz < f ≤ 6GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |  |  |
| 7.8.1G.3 Wide band Intermodulation for V2X Communication / Intra-band contiguous multi-carrier operation |  | Intermodulation ±3.8 dB, 4.2GHz < f ≤ 6GHz |  | Overall intermodulation uncertainty comprises three quantities: 1. Wanted signal level error 2. CW Interferer level error 3. Modulated Interferer level error Effect of interferer ACLR has not been included as modulated interferer has larger frequency offset The effect of the closer CW signal has twice the effect. Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared to provide the combined effect of the three signals. Test System uncertainty = SQRT [(2 x CW_level_error)2 +(mod interferer_level_error)2 +(wanted signal_level_error)2] 4.2GHz < f ≤ 6GHz Wanted signal level ± 1.5dB CW Interferer level ± 1.3dB Mod Interferer level ± 1.5dB |  |
| 7.9 Spurious emissions |  | 30MHz ≤ f ≤ 4.0GHz: ± 2.0 dB 4 GHz < f ≤ 19 GHz: ± 4.0 dB |  |  |  |
| 7.9_1 Spurious emissions with 4 Rx antenna ports |  | Same as 7.9 |  |  |  |
| 7.9A Spurious emissions for CA |  | 30MHz ≤ f ≤ 4.0GHz: ± 2.0 dB 4 GHz < f ≤ 19 GHz: ± 4.0 dB 19GHz < f ≤ 26 GHz: ± 6.0 dB |  |  |  |
| 7.9E Spurious emissions for UE category 0 |  | Same as 7.9 |  |  |  |
| 7.9EA Spurious emissions for UE category M1 |  | Same as 7.9 |  |  |  |
| 7.9EB Spurious emissions for UE Category 1bis |  | Same as 7.9 |  |  |  |
| 7.9EC Spurious emissions for UE category M2 |  | Same as 7.9 |  |  |  |
| 7.9F Spurious emissions for category NB1 and NB2 |  | Same as 7.9 |  |  |  |
| 7.9G.1 Spurious emissions for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | 30MHz ≤ f ≤ 4.0GHz: ± 2.0 dB 4 GHz < f ≤ 19 GHz: ± 4.0 dB 19GHz < f ≤ 26 GHz: ± 6.0 dB |  |  |  |
| Note 1: Unless otherwise noted, only the Test System stimulus error is considered here. The effect of errors in the throughput measurements due to finite test duration is not considered. |  |  |  |  |  |

## F.1.4	Measurement of performance requirements

Table F.1.4-1: Maximum Test System Uncertainty for Performance Requirements

| Subclause |  |  | Maximum Test System Uncertainty1 |  |  | Derivation of Test System Uncertainty |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.2.1.1.1 Multiple PRBs - Propagation Condition EVA5 - Propagation Condition ETU70 - Propagation Condition ETU300 |  |  | ± 0.8 dB |  |  | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB |  |  |
| 8.2.1.1.1 Multiple PRBs - Propagation Condition HST |  |  | ± 0.6 dB |  |  | Overall system uncertainty for HST condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |  |
| 8.2.1.1.1 Single PRB - Propagation Condition ETU70 |  |  | ± 0.8 dB |  |  | Overall system uncertainty for fading condition comprises three quantities: 1. Average Signal-to-noise ratio uncertainty 2. Signal-to noise ratio variation for single PRB 3. Fading profile power uncertainty Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Average signal-to-noise ratio uncertainty 2 + Signal-to-noise ratio variation2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Signal-to-noise ratio variation ±0.5 dB Fading profile power uncertainty ±0.5 dB for single Tx |  |  |
| 8.2.1.1.1_A.1 |  |  | Same as 8.2.1.1.1 Multiple PRBs, for each CC |  |  | Same as 8.2.1.1.1 Multiple PRBs Calculation applies for each CC |  |  |
| 8.2.1.1.1_A.2 |  |  | Same as 8.2.1.1.1 Multiple PRBs, for each CC |  |  | Same as 8.2.1.1.1 Multiple PRBs Calculation applies for each CC |  |  |
| 8.2.1.1.1_A.4 |  |  | Same as 8.2.1.1.1 Multiple PRBs, for each CC |  |  | Same as 8.2.1.1.1 Multiple PRBs Calculation applies for each CC |  |  |
| 8.2.1.1.1_A.5 |  |  | Same as 8.2.1.1.1 Multiple PRBs, for each CC |  |  | Same as 8.2.1.1.1 Multiple PRBs Calculation applies for each CC |  |  |
| 8.2.1.1.1_A.6 |  |  | Same as 8.2.1.1.1 Multiple PRBs, for each CC |  |  | Same as 8.2.1.1.1 Multiple PRBs Calculation applies for each CC |  |  |
| 8.2.1.1.1_A.7 |  |  | Same as 8.2.1.1.1 Multiple PRBs, for each CC |  |  | Same as 8.2.1.1.1 Multiple PRBs Calculation applies for each CC |  |  |
| 8.2.1.3.1_A.5 |  |  | Same as 8.2.1.3.1 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.2.1.1.1_1 Multiple PRBs - Propagation Condition EVA5 - Propagation Condition ETU70 - Propagation Condition ETU300 |  |  | Same as 8.2.1.1.1 Multiple PRBs Propagation EVA5, ETU70, ETU300 |  |  |  |  |  |
| 8.2.1.1.1_2 Multiple PRB - Propagation Condition EVA5 |  |  | Same as 8.2.1.1.1 Multiple PRBs Propagation EVA5 |  |  |  |  |  |
| 8.2.1.1.2 Single PRB |  |  | ± 0.8 dB |  |  | Same as 8.2.1.1.1 Single PRB |  |  |
| 8.2.1.2.1 - Propagation Condition EVA5 |  |  | ± 0.9 dB |  |  | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB |  |  |
| 8.2.1.2.1 - Propagation Condition HST |  |  | ± 0.6 dB |  |  | Overall system uncertainty for HST condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |  |
| 8.2.1.2.1_1 - Propagation Condition EVA5 |  |  | Same as 8.2.1.2.1 Propagation EVA5 |  |  |  |  |  |
| 8.2.1.2.2 |  |  | ± 0.9 dB |  |  | Same as 8.2.1.2.1 Propagation Condition EVA5 |  |  |
| 8.2.1.2.2_1 |  |  | Same as 8.2.1.2.2 |  |  |  |  |  |
| 8.2.1.2.3_C.1 |  |  | Symbols #0, 4.. during ABS: Ês1/Noc2 ± 0.9 dB Ês2/ Ês1 ± 1.1 dB Symbols #1, 2.. during ABS: Ês1/Noc1 ± 1.0 dB All symbols during non-ABS: Ês1/Noc3 ± 1.0 dB |  |  | System uncertainties comprise a number of quantities which are selected according to their applicability in the equations below. Uncertainties are assumed to be uncorrelated, so can be root sum squared. AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Symbols #0, 4.. means 0, 4, 7, 11 Symbols #1, 2.. means 1-3, 5, 6, 8-10, 12, 13 Symbols #0, 4.. during ABS: Ês1/Noc2 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Ês2/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 2 SNR uncertainty 2 + Cell 2 Fading profile power uncertainty 2) Symbols #1, 2.. during ABS: Ês1/Noc1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc1/Noc2 uncertainty 2) All symbols during non-ABS: Ês1/Noc3 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc3/Noc2 uncertainty 2) Component uncertainties: Cell 1 SNR uncertainty (Es1 / Noc2 ratio before fading) ±0.3 dB Cell 2 SNR uncertainty (Es2 / Noc2 ratio before fading) ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB Noc1/Noc2 and Noc3/Noc2 uncertainty ±0.3 dB |  |  |
| 8.2.1.2.3_E.1 - Propagation Condition EVA5 |  |  | Symbols #0, 4.. during ABS: Ês1/Noc2 ± 0.9dB Ês2/ Ês1 ± 1.1dB Ês3/ Ês1 ± 1.1dB Symbols #1, 2.. during ABS: Ês1/Noc1 ± 1.0dB All symbols during non-ABS: Ês1/Noc3 ± 1.0dB |  |  | System uncertainties comprise a number of quantities which are selected according to their applicability in the equations below. Uncertainties are assumed to be uncorrelated, so can be root sum squared. AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Symbols #0, 4.. means 0, 4, 7, 11 Symbols #1, 2.. means 1-3, 5, 6, 8-10, 12, 13 Symbols #0, 4.. during ABS: Ês1/Noc2 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Ês2/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 2 SNR uncertainty 2 + Cell 2 Fading profile power uncertainty 2) Ês3/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 3 SNR uncertainty 2 + Cell 3 Fading profile power uncertainty 2) Symbols #1, 2.. during ABS: Ês1/Noc1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc1/Noc2 uncertainty 2) All symbols during non-ABS: Ês1/Noc3 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc3/Noc2 uncertainty 2) Component uncertainties: SNR uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB Noc1/Noc2 and Noc3/Noc2 uncertainty ±0.3 dB |  |  |
| 8.2.1.2.4 |  |  | Ês/Noc ± 0.8 dB for each cell |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |  |
| 8.2.1.2.5 |  |  | Ês/Noc ± 0.8 dB for each cell |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |  |
| 8.2.1.2.6 |  |  | Same as 8.2.1.2.5 |  |  | Same as 8.2.1.2.5 |  |  |
| 8.2.1.3.1 |  |  | ± 0.9 dB |  |  | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB |  |  |
| 8.2.1.3.1_1 |  |  | Same as 8.2.1.3.1 |  |  | Same as 8.2.1.3.1 |  |  |
| 8.2.1.3.1_A.1 |  |  | Same as 8.2.1.3.1 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.2.1.3.1_A.2 FDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (3DL CA) |  |  | Same as 8.2.1.3.1 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.2.1.3.1_A.3 FDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (4DL CA) |  |  | Same as 8.2.1.3.1 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.2.1.3.1_A.3 |  |  | Same as 8.2.1.3.1 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.2.1.3.1_A.4 |  |  | Same as 8.2.1.3.1 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.2.1.3.1_A.5 |  |  | Same as 8.2.1.3.1 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.2.1.3.1_A.6 |  |  | Same as 8.2.1.3.1 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.2.1.3.1A_A.1 |  |  | Same as 8.2.1.3.1 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.2.1.3.1B |  |  | ± 0.9 dB |  |  | Same as 8.2.1.3.1 |  |  |
| 8.2.1.3.1C |  |  | Ês/Noc ± 0.8 dB for each cell |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |  |
| 8.2.1.3.2 |  |  | ± 0.9 dB |  |  | Same as 8.2.1.3.1 |  |  |
| 8.2.1.3.3_C.1 |  |  | Symbols #0, 4.. during ABS: Ês1/Noc2 ± 0.9 dB Ês2/ Ês1 ± 1.1 dB Symbols #1, 2.. during ABS: Ês1/Noc1 ± 1.0 dB All symbols during non-ABS: Ês1/Noc3 ± 1.0 dB |  |  | System uncertainties comprise a number of quantities which are selected according to their applicability in the equations below. Uncertainties are assumed to be uncorrelated, so can be root sum squared. AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Symbols #0, 4.. means 0, 4, 7, 11 Symbols #1, 2.. means 1-3, 5, 6, 8-10, 12, 13 Symbols #0, 4.. during ABS: Ês1/Noc2 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Ês2/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 2 SNR uncertainty 2 + Cell 2 Fading profile power uncertainty 2) Symbols #1, 2.. during ABS: Ês1/Noc1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc1/Noc2 uncertainty 2) All symbols during non-ABS: Ês1/Noc3 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc3/Noc2 uncertainty 2) Component uncertainties: Cell 1 SNR uncertainty (Es1 / Noc2 ratio before fading) ±0.3 dB Cell 2 SNR uncertainty (Es2 / Noc2 ratio before fading) ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB Noc1/Noc2 and Noc3/Noc2 uncertainty ±0.3 dB |  |  |
| 8.2.1.3.3_C.2 |  |  | Symbol #0 during ABS: Ês1/Noc2 ± 0.9 dB Ês2/ Ês1 ± 1.1 dB Symbols #1..13 during ABS: Ês1/Noc1 ± 1.0 dB All symbols during non-ABS: Ês1/Noc3 ± 1.0 dB |  |  | System uncertainties comprise a number of quantities which are selected according to their applicability in the equations below. Uncertainties are assumed to be uncorrelated, so can be root sum squared. AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Symbol #0 during ABS: Ês1/Noc2 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Ês2/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 2 SNR uncertainty 2 + Cell 2 Fading profile power uncertainty 2) Symbols #1..13 during ABS: Ês1/Noc1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc1/Noc2 uncertainty 2) All symbols during non-ABS: Ês1/Noc3 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc3/Noc2 uncertainty 2) Component uncertainties: Cell 1 SNR uncertainty (Es1 / Noc2 ratio before fading) ±0.3 dB Cell 2 SNR uncertainty (Es2 / Noc2 ratio before fading) ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB Noc1/Noc2 and Noc3/Noc2 uncertainty ±0.3 dB |  |  |
| 8.2.1.3.3_E.1 - Propagation Condition EVA5 |  |  | Symbols #0, 4.. during ABS: Ês1/Noc2 ± 0.9dB Ês2/ Ês1 ± 1.1dB Ês3/ Ês1 ± 1.1dB Symbols #1, 2.. during ABS: Ês1/Noc1 ± 1.0dB All symbols during non-ABS: Ês1/Noc3 ± 1.0dB |  |  | System uncertainties comprise a number of quantities which are selected according to their applicability in the equations below. Uncertainties are assumed to be uncorrelated, so can be root sum squared. AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Symbols #0, 4.. means 0, 4, 7, 11 Symbols #1, 2.. means 1-3, 5, 6, 8-10, 12, 13 Symbols #0, 4.. during ABS: Ês1/Noc2 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Ês2/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 2 SNR uncertainty 2 + Cell 2 Fading profile power uncertainty 2) Ês3/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 3 SNR uncertainty 2 + Cell 3 Fading profile power uncertainty 2) Symbols #1, 2.. during ABS: Ês1/Noc1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc1/Noc2 uncertainty 2) All symbols during non-ABS: Ês1/Noc3 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc3/Noc2 uncertainty 2) Component uncertainties: SNR uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB Noc1/Noc2 and Noc3/Noc2 uncertainty ±0.3 dB |  |  |
| 8.2.1.4.1 |  |  | ± 0.9 dB |  |  | Same as 8.2.1.3.1 |  |  |
| 8.2.1.4.1_1 |  |  | Same as 8.2.1.4.1 |  |  |  |  |  |
| 8.2.1.4.1_E.1 - Propagation Condition EPA5 |  |  | Symbols #0, 4.. during ABS: Ês1/Noc2 ± 0.9 dB Ês2/ Ês1 ± 1.1 dB Ês3/ Ês1 ± 1.1 dB Symbols #1, 2.. during ABS: Ês1/Noc1 ± 1.0 dB All symbols during non-ABS: Ês1/Noc3 ± 1.0 dB |  |  | System uncertainties comprise a number of quantities which are selected according to their applicability in the equations below. Uncertainties are assumed to be uncorrelated, so can be root sum squared. AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Symbols #0, 4.. means 0, 4, 7, 11 Symbols #1, 2.. means 1-3, 5, 6, 8-10, 12, 13 Symbols #0, 4.. during ABS: Ês1/Noc2 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Ês2/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 2 SNR uncertainty 2 + Cell 2 Fading profile power uncertainty 2) Ês3/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 3 SNR uncertainty 2 + Cell 3 Fading profile power uncertainty 2) Symbols #1, 2.. during ABS: Ês1/Noc1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc1/Noc2 uncertainty 2) All symbols during non-ABS: Ês1/Noc3 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc3/Noc2 uncertainty 2) Component uncertainties: SNR uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB Noc1/Noc2 and Noc3/Noc2 uncertainty ±0.3 dB |  |  |
| 8.2.1.4.1_H FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 2x2 for SCE-L1 |  |  | ± 0.9 dB |  |  | Same as 8.2.1.4.1 |  |  |
| 8.2.1.4.2 |  |  | ± 0.9 dB |  |  | Same as 8.2.1.3.1 |  |  |
| 8.2.1.4.2_1 |  |  | Same as 8.2.1.4.2 |  |  |  |  |  |
| 8.2.1.4.2_A.1 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.2.1.4.2_A.2 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.2.1.4.2_A.3 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.2.1.4.2_A.4 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.2.1.4.2_A.5 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.2.1.4.2_A.6 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.2.1.4.2A |  |  | ± 0.9 dB |  |  | Same as 8.2.1.3.1 |  |  |
| 8.2.1.4.3 |  |  | Ês/Noc ± 0.8 dB for each cell |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |  |
| 8.2.1.4.3A |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.2.1.4.4 |  |  | Ês/Noc ± 0.8 dB for each cell |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |  |
| 8.2.1.7.1_A.1 |  |  | TBD |  |  | TBD |  |  |
| 8.2.1.9 - Propagation Condition HST-SFN |  |  | ± 0.6 dB |  |  | Overall system uncertainty for HST-SFN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |  |
| 8.2.1.10 - Propagation Condition EVA5 |  |  | ± 0.8 dB |  |  | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB |  |  |
| 8.2.2.1.1 Multiple PRBs - Propagation Condition EVA5 - Propagation Condition ETU70 - Propagation Condition ETU300 |  |  | ± 0.8 dB |  |  | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB |  |  |
| 8.2.2.1.1 Multiple PRBs - Propagation Condition HST |  |  | ± 0.6 dB |  |  | Overall system uncertainty for HST condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |  |
| 8.2.2.1.1 Single PRB - Propagation Condition ETU70 |  |  | ± 0.8 dB |  |  | Overall system uncertainty for fading condition comprises three quantities: 1. Average Signal-to-noise ratio uncertainty 2. Signal-to noise ratio variation for single PRB 3. Fading profile power uncertainty Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Average signal-to-noise ratio uncertainty 2 + Signal-to-noise ratio variation2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Signal-to-noise ratio variation ±0.5 dB Fading profile power uncertainty ±0.5 dB for single Tx |  |  |
| 8.2.2.1.1_1 Multiple PRBs - Propagation Condition EVA5 - Propagation Condition ETU70 - Propagation Condition ETU300 |  |  | Same as 8.2.2.1.1 Multiple PRBs Propagation EVA5, ETU70, ETU300 |  |  |  |  |  |
| 8.2.2.1.1_2 Multiple PRB - Propagation Condition EVA5 |  |  | Same as 8.2.2.1.1 Multiple PRBs Propagation EVA5 |  |  |  |  |  |
| 8.2.2.1.1_A.1 |  |  | Same as 8.2.2.1.1 Multiple PRBs for each CC |  |  | Same as 8.2.2.1.1 Multiple PRBs Calculation applies for each CC |  |  |
| 8.2.2.1.1_A.2 |  |  | Same as 8.2.2.1.1 Multiple PRBs for each CC |  |  | Same as 8.2.2.1.1 Multiple PRBs Calculation applies for each CC |  |  |
| 8.2.2.1.1_A.3 |  |  | Same as 8.2.2.1.1 Multiple PRBs for each CC |  |  | Same as 8.2.2.1.1 Multiple PRBs Calculation applies for each CC |  |  |
| 8.2.2.1.1_A.4 |  |  | Same as 8.2.2.1.1 Multiple PRBs for each CC |  |  | Same as 8.2.2.1.1 Multiple PRBs Calculation applies for each CC |  |  |
| 8.2.2.1.2 Single PRB |  |  | ± 0.8 dB |  |  | Same as 8.2.2.1.1 Single PRB |  |  |
| 8.2.2.2.1 - Propagation Condition EVA5 |  |  | ± 0.9 dB |  |  | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB |  |  |
| 8.2.2.2.1 - Propagation Condition HST |  |  | ± 0.6 dB |  |  | Overall system uncertainty for HST condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |  |
| 8.2.2.2.1_1 - Propagation Condition EVA5 |  |  | Same as 8.2.2.2.1 Propagation EVA5 |  |  |  |  |  |
| 8.2.2.2.2 |  |  | ± 0.9 dB |  |  | Same as 8.2.2.2.1 Propagation Condition EVA5 |  |  |
| 8.2.2.2.2_1 |  |  | Same as 8.2.2.2.2 |  |  |  |  |  |
| 8.2.2.2.3_C.1 |  |  | Same as 8.2.1.2.3_C.1 |  |  | Same as 8.2.1.2.3_C.1 |  |  |
| 8.2.2.2.3_E.1 - Propagation Condition EVA5 |  |  | Same as 8.2.1.2.3_E.1 |  |  | Same as 8.2.1.2.3_E.1 |  |  |
| 8.2.2.2.4 |  |  | Ês/Noc ± 0.8 dB for each cell |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |  |
| 8.2.2.2.5 |  |  | ± 0.9 dB |  |  | Same as 8.2.2.2.1 Propagation Condition EVA5 |  |  |
| 8.2.2.2.6 |  |  | Ês/Noc ± 0.8 dB for each cell |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |  |
| 8.2.2.2.7 |  |  | Same as 8.2.2.2.6 |  |  | Same as 8.2.2.2.6 |  |  |
| 8.2.2.3.1 |  |  | ± 0.9 dB |  |  | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB |  |  |
| 8.2.2.3.1_1 |  |  | Same as 8.2.2.3.1 |  |  | Same as 8.2.2.3.1 |  |  |
| 8.2.2.3.1_A.1 |  |  | Same as 8.2.2.3.1 for each CC |  |  | Same as 8.2.2.3.1 Calculation applies for each CC |  |  |
| 8.2.2.3.1_A.2 TDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (3 DL CA) |  |  | Same as 8.2.2.3.1 for each CC |  |  | Same as 8.2.2.3.1 Calculation applies for each CC |  |  |
| 8.2.2.3.1_A.4 TDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (5 DL CA) |  |  | Same as 8.2.2.3.1 for each CC |  |  | Same as 8.2.2.3.1 Calculation applies for each CC |  |  |
| 8.2.2.3.1A_A.1 |  |  | Same as 8.2.2.3.1 for each CC |  |  | Same as 8.2.2.3.1 Calculation applies for each CC |  |  |
| 8.2.2.3.1B |  |  | ± 0.9 dB |  |  | Same as 8.2.2.3.1 |  |  |
| 8.2.2.3.1C |  |  | Same as 8.2.1.3.1C |  |  | Same as 8.2.1.3.1C |  |  |
| 8.2.2.3.2 |  |  | ± 0.9 dB |  |  | Same as 8.2.2.3.1 |  |  |
| 8.2.2.3.3_C.1 |  |  | Same as 8.2.1.3.3_C.1 |  |  | Same as 8.2.1.3.3_C.1 |  |  |
| 8.2.2.3.3_C.2 |  |  | Same as 8.2.1.3.3_C.2 |  |  | Same as 8.2.1.3.3_C.2 |  |  |
| 8.2.2.3.3_E.1 - Propagation Condition EVA5 |  |  | Same as 8.2.1.3.3_E.1 |  |  | Same as 8.2.1.3.3_E.1 |  |  |
| 8.2.2.4.1 |  |  | ± 0.9 dB |  |  | Same as 8.2.2.3.1 |  |  |
| 8.2.2.4.1_1 |  |  | Same as 8.2.2.4.1 |  |  |  |  |  |
| 8.2.2.4.1_E.1 - Propagation Condition EPA5 |  |  | Same as 8.2.1.4.1_E.1 |  |  | Same as 8.2.1.4.1_E.1 |  |  |
| 8.2.2.4.1_H TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 2x2 for SCE-L1 |  |  | Same as 8.2.2.4.1 |  |  |  |  |  |
| 8.2.2.4.2 |  |  | ± 0.9 dB |  |  | Same as 8.2.2.3.1 |  |  |
| 8.2.2.4.2_1 |  |  | Same as 8.2.2.4.2 |  |  |  |  |  |
| 8.2.2.4.2_A.1 |  |  | Same as 8.2.2.4.2 for each CC |  |  | Same as 8.2.2.3.1 Calculation applies for each CC |  |  |
| 8.2.2.4.2_A.2 TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (3DL CA) |  |  | Same as 8.2.2.4.2 for each CC |  |  | Same as 8.2.2.3.1 Calculation applies for each CC |  |  |
| 8.2.2.4.2_A.3 TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (4DL CA) |  |  | Same as 8.2.2.4.2 for each CC |  |  | Same as 8.2.2.3.1 Calculation applies for each CC |  |  |
| 8.2.2.4.2_A.4 TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (5DL CA) |  |  | Same as 8.2.2.4.2 for each CC |  |  | Same as 8.2.2.3.1 Calculation applies for each CC |  |  |
| 8.2.2.4.2A |  |  | ± 0.9 dB |  |  | Same as 8.2.2.3.1 |  |  |
| 8.2.2.4.3 |  |  | Ês/Noc ± 0.8 dB for each cell |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |  |
| 8.2.2.4.4 |  |  | Same as 8.2.2.4.2 for each CC |  |  | Same as 8.2.2.3.1 Calculation applies for each CC |  |  |
| 8.2.2.4.5 |  |  | Ês/Noc ± 0.8 dB for each cell |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |  |
| 8.2.2.7.1_A.1 |  |  | TBD |  |  | TBD |  |  |
| 8.2.2.8.1 |  |  | Same as 8.2.2.1.1 Multiple PRBs for each CC - Propagation Condition EVA5 |  |  | Same as 8.2.2.1.1 Multiple PRBs with Propagation Condition EVA5 Calculation applies for each CC |  |  |
| 8.2.2.8.2 |  |  | Same as 8.2.2.1.1 Multiple PRBs for each CC - Propagation Condition EVA5 |  |  | Same as 8.2.2.1.1 Multiple PRBs with Propagation Condition EVA5 Calculation applies for each CC |  |  |
| 8.2.2.9 - Propagation Condition HST-SFN |  |  | ± 0.6 dB |  |  | Overall system uncertainty for HST-SFN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |  |
| 8.2.3.1.1.1 |  |  | Same as 8.2.1.1.1 Multiple PRBs, for each CC |  |  | Same as 8.2.1.1.1 Multiple PRBs Calculation applies for each CC |  |  |
| 8.2.3.1.1.2 |  |  | Same as 8.2.1.1.1 Multiple PRBs, for each CC |  |  | Same as 8.2.1.1.1 Multiple PRBs Calculation applies for each CC |  |  |
| 8.2.3.1.1.3 |  |  | Same as 8.2.1.1.1 Multiple PRBs, for each CC |  |  | Same as 8.2.1.1.1 Multiple PRBs Calculation applies for each CC |  |  |
| 8.2.3.1.1.4 |  |  | Same as 8.2.1.1.1 Multiple PRBs, for each CC |  |  | Same as 8.2.1.1.1 Multiple PRBs Calculation applies for each CC |  |  |
| 8.2.3.1.1.5 |  |  | Same as 8.2.1.1.1 Multiple PRBs, for each CC |  |  | Same as 8.2.1.1.1 Multiple PRBs Calculation applies for each CC |  |  |
| 8.2.3.1.1.6 |  |  | Same as 8.2.1.1.1 Multiple PRBs, for each CC |  |  | Same as 8.2.1.1.1 Multiple PRBs Calculation applies for each CC |  |  |
| 8.2.3.1.2.1 |  |  | Same as 8.2.1.1.1 Multiple PRBs, for each CC |  |  | Same as 8.2.1.1.1 Multiple PRBs Calculation applies for each CC |  |  |
| 8.2.3.1.2.2 |  |  | Same as 8.2.1.1.1 Multiple PRBs, for each CC |  |  | Same as 8.2.1.1.1 Multiple PRBs Calculation applies for each CC |  |  |
| 8.2.3.1.2.3 |  |  | Same as 8.2.1.1.1 Multiple PRBs, for each CC |  |  | Same as 8.2.1.1.1 Multiple PRBs Calculation applies for each CC |  |  |
| 8.2.3.1.2.4 |  |  | Same as 8.2.1.1.1 Multiple PRBs, for each CC |  |  | Same as 8.2.1.1.1 Multiple PRBs Calculation applies for each CC |  |  |
| 8.2.3.1.2.5 |  |  | Same as 8.2.1.1.1 Multiple PRBs, for each CC |  |  | Same as 8.2.1.1.1 Multiple PRBs Calculation applies for each CC |  |  |
| 8.2.3.1.2.6 |  |  | Same as 8.2.1.1.1 Multiple PRBs, for each CC |  |  | Same as 8.2.1.1.1 Multiple PRBs Calculation applies for each CC |  |  |
| 8.2.3.2.1.1 |  |  | Same as 8.2.1.3.1 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.2.3.2.1.2 |  |  | Same as 8.2.1.3.1 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.2.3.2.1.3 |  |  | Same as 8.2.1.3.1 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.2.3.2.1.4 |  |  | Same as 8.2.1.3.1 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.2.3.2.1.5 |  |  | Same as 8.2.1.3.1 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.2.3.2.1.6 |  |  | Same as 8.2.1.3.1 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.2.3.2.1A |  |  | Same as 8.2.1.3.1 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.2.3.2.2.1 |  |  | Same as 8.2.2.3.1 for each CC |  |  | Same as 8.2.2.3.1 Calculation applies for each CC |  |  |
| 8.2.3.2.2.2 |  |  | Same as 8.2.2.3.1 for each CC |  |  | Same as 8.2.2.3.1 Calculation applies for each CC |  |  |
| 8.2.3.2.2.3 |  |  | Same as 8.2.2.3.1 for each CC |  |  | Same as 8.2.2.3.1 Calculation applies for each CC |  |  |
| 8.2.3.2.2.4 |  |  | Same as 8.2.2.3.1 for each CC |  |  | Same as 8.2.2.3.1 Calculation applies for each CC |  |  |
| 8.2.3.2.2.5 |  |  | Same as 8.2.2.3.1 for each CC |  |  | Same as 8.2.2.3.1 Calculation applies for each CC |  |  |
| 8.2.3.2.2.6 |  |  | Same as 8.2.2.3.1 for each CC |  |  | Same as 8.2.2.3.1 Calculation applies for each CC |  |  |
| 8.2.3.2.2A |  |  | Same as 8.2.2.3.1 for each CC |  |  | Same as 8.2.2.3.1 Calculation applies for each CC |  |  |
| 8.2.3.3.1.1 |  |  | Same as 8.2.1.3.1 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.2.3.3.1.2 |  |  | Same as 8.2.1.3.1 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.2.3.3.1.3 |  |  | Same as 8.2.1.3.1 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.2.3.3.1.4 |  |  | Same as 8.2.1.3.1 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.2.3.3.1.5 |  |  | Same as 8.2.1.3.1 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.2.3.3.1.6 |  |  | Same as 8.2.1.3.1 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.2.3.3.2.1 |  |  | Same as 8.2.2.3.1 for each CC |  |  | Same as 8.2.2.3.1 Calculation applies for each CC |  |  |
| 8.2.3.3.2.2 |  |  | Same as 8.2.2.3.1 for each CC |  |  | Same as 8.2.2.3.1 Calculation applies for each CC |  |  |
| 8.2.3.3.2.3 |  |  | Same as 8.2.2.3.1 for each CC |  |  | Same as 8.2.2.3.1 Calculation applies for each CC |  |  |
| 8.2.3.3.2.4 |  |  | Same as 8.2.2.3.1 for each CC |  |  | Same as 8.2.2.3.1 Calculation applies for each CC |  |  |
| 8.2.3.3.2.5 |  |  | Same as 8.2.1.3.1 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.2.3.3.2.6 |  |  | Same as 8.2.1.3.1 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.2.4.1.1 |  |  | ± 0.9 dB |  |  | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB |  |  |
| 8.2.4.1.2 |  |  | Same as 8.2.4.1.1 for each CC |  |  | Same as 8.2.4.1.1 Calculation applies for each CC |  |  |
| 8.3.1.1.1_D |  |  | ± 0.9 dB |  |  | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB |  |  |
| 8.3.1.1.1_H FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 without a simultaneous transmission for eDL-MIMO for SCE-L1 |  |  | Same as 8.3.1.1.1_D |  |  | Same as 8.3.1.1.1_D |  |  |
| 8.3.1.1.2_D |  |  | ± 0.9 dB |  |  | Same as 8.3.1.1.1_D |  |  |
| 8.3.1.1.3 |  |  | Ês/Noc ± 0.8 dB for each cell |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |  |
| 8.3.1.1.4 |  |  | Ês/Noc ± 0.8 dB for each cell |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |  |
| 8.3.1.1.5 |  |  | Ês1/Noc2 ± 0.9 dB Ês2/ Ês1 ± 1.1 dB |  |  | System uncertainties comprise a number of quantities which are selected according to their applicability in the equations below. Uncertainties are assumed to be uncorrelated, so can be root sum squared. AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Ês1/Noc2 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Ês2/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 2 SNR uncertainty 2 + Cell 2 Fading profile power uncertainty 2) Component uncertainties: Cell 1 SNR uncertainty (Es1 / Noc ratio before fading) ±0.3 dB Cell 2 SNR uncertainty (Es2 / Noc ratio before fading) ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB |  |  |
| 8.3.1.1.6 |  |  | Ês/Noc ± 0.8 dB for each cell |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |  |
| 8.3.1.1.7 |  |  | TBD |  |  | TBD |  |  |
| 8.3.1.1.9 |  |  | ± 0.9 dB |  |  | Same as 8.3.1.1.1_D |  |  |
| 8.3.1.2.1_D |  |  | ± 0.9 dB |  |  | Same as 8.3.1.1.1_D |  |  |
| 8.3.1.2.1_D_1 |  |  | Ês/Noc ± 0.8 dB for each cell |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |  |
| 8.3.1.2.2 |  |  | ± 0.9 dB |  |  | Same as 8.3.1.1.1_D |  |  |
| 8.3.1.3.1_F |  |  | Ês/Noc ± 0.9 dB for each TP Time alignment error TP 2 relative to TP1: ±130 ns (±4Ts) |  |  | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB |  |  |
| 8.3.1.3.2_F |  |  | Ês/Noc ± 0.9 dB for each TP Time alignment error TP 2 relative to TP1: ±130 ns (±4Ts) |  |  | Same as 8.3.1.3.1_F |  |  |
| 8.3.1.3.3_F |  |  | Ês/Noc ± 0.9 dB for each TP |  |  | Same as 8.3.1.3.1_F |  |  |
| 8.3.2.1.1 |  |  | ± 0.9 dB |  |  | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB |  |  |
| 8.3.2.1.1_1 |  |  | ± 0.9 dB |  |  | Same as 8.3.2.1.1 |  |  |
| 8.3.2.1.2 |  |  | ± 0.9 dB |  |  | Same as 8.2.2.3.1 |  |  |
| 8.3.2.1.2_D |  |  | ± 0.9 dB |  |  | Same as 8.2.2.3.1 |  |  |
| 8.3.2.1.2_H TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 without a simultaneous transmission for eDL-MIMO for SCE-L1 |  |  | ± 0.9 dB |  |  | Same as 8.2.2.3.1 |  |  |
| 8.3.2.1.3 |  |  | ± 0.9 dB |  |  | Same as 8.2.2.3.1 |  |  |
| 8.3.2.1.3_D |  |  | ± 0.9 dB |  |  | Same as 8.2.2.3.1 |  |  |
| 8.3.2.1.4 |  |  | Ês/Noc ± 0.8 dB for each cell |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |  |
| 8.3.2.1.5 |  |  | Ês/Noc ± 0.8 dB for each cell |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |  |
| 8.3.2.1.6 |  |  | Ês1/Noc2 ± 0.9 dB Ês2/ Ês1 ± 1.1 dB |  |  | System uncertainties comprise a number of quantities which are selected according to their applicability in the equations below. Uncertainties are assumed to be uncorrelated, so can be root sum squared. AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Ês1/Noc2 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Ês2/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 2 SNR uncertainty 2 + Cell 2 Fading profile power uncertainty 2) Component uncertainties: Cell 1 SNR uncertainty (Es1 / Noc ratio before fading) ±0.3 dB Cell 2 SNR uncertainty (Es2 / Noc ratio before fading) ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB |  |  |
| 8.3.2.1.7 |  |  | Ês/Noc ± 0.8 dB for each cell |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |  |
| 8.3.2.1.8 |  |  | TBD |  |  | TBD |  |  |
| 8.3.2.1.10 |  |  | ± 0.9 dB |  |  | Same as 8.2.2.3.1 |  |  |
| 8.3.2.2.1 |  |  | ± 0.9 dB |  |  | Same as 8.2.2.3.1 |  |  |
| 8.3.2.2.1_D |  |  | ± 0.9 dB |  |  | Same as 8.2.2.3.1 |  |  |
| 8.3.2.2.1_D_1 |  |  | Ês/Noc ± 0.8 dB for each cell |  |  | Same as 8.3.1.2.1_D_1 |  |  |
| 8.3.2.2.2 |  |  | ± 0.9 dB |  |  | Same as 8.2.2.3.1 |  |  |
| 8.3.2.4.1_F |  |  | Ês/Noc ± 0.9 dB for each TP Time alignment error TP 2 relative to TP1: ±130 ns (±4Ts) |  |  | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB |  |  |
| 8.3.2.4.2_F |  |  | Ês/Noc ± 0.9 dB for each TP Time alignment error TP 2 relative to TP1: ±130 ns (±4Ts) |  |  | Same as 8.3.2.4.1_F |  |  |
| 8.3.2.4.3_F |  |  | Ês/Noc ± 0.9 dB for each TP |  |  | Same as 8.3.2.4.1_F |  |  |
| 8.3.3.1.1 |  |  | ± 0.9 dB |  |  | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB |  |  |
| 8.3.3.1.2 |  |  | Same as 8.3.3.1.1 |  |  | Same as 8.3.3.1.1 |  |  |
| 8.4.1.1 |  |  | ± 0.8 dB |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.2 dB |  |  |
| 8.4.1.2.1 |  |  | ± 1.0 dB |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.4 dB |  |  |
| 8.4.1.2.1_1 |  |  | Same as 8.4.1.2.1 |  |  |  |  |  |
| 8.4.1.2.2 |  |  | ± 1.0 dB |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.4 dB |  |  |
| 8.4.1.2.2_1 |  |  | Same as 8.4.1.2.2 |  |  |  |  |  |
| 8.4.1.2.3_C.1 |  |  | Symbols #0, 4.. during ABS: Ês1/Noc2 ± 0.9 dB Ês2/ Ês1 ± 1.1 dB Symbols #1, 2.. during ABS: Ês1/Noc1 ± 1.0 dB All symbols during non-ABS: Ês1/Noc3 ± 1.0 dB |  |  | System uncertainties comprise a number of quantities which are selected according to their applicability in the equations below. Uncertainties are assumed to be uncorrelated, so can be root sum squared. AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Symbols #0, 4.. means 0, 4, 7, 11 Symbols #1, 2.. means 1-3, 5, 6, 8-10, 12, 13 Symbols #0, 4.. during ABS: Ês1/Noc2 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Ês2/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 2 SNR uncertainty 2 + Cell 2 Fading profile power uncertainty 2) Symbols #1, 2.. during ABS: Ês1/Noc1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc1/Noc2 uncertainty 2) All symbols during non-ABS: Ês1/Noc3 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc3/Noc2 uncertainty 2) Component uncertainties: SNR uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB Noc1/Noc2 and Noc3/Noc2 uncertainty ±0.3 dB |  |  |
| 8.4.1.2.3_C.2 |  |  | Symbols #0, 4.. during ABS: Ês1/Noc2 ± 0.9 dB Ês2/ Ês1 ± 1.1 dB Symbols #1, 2.. during ABS: Ês1/Noc1 ± 1.0 dB All symbols during non-ABS: Ês1/Noc3 ± 1.0 dB |  |  | System uncertainties comprise a number of quantities which are selected according to their applicability in the equations below. Uncertainties are assumed to be uncorrelated, so can be root sum squared. AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Symbols #0, 4.. means 0, 4, 7, 11 Symbols #1, 2.. means 1-3, 5, 6, 8-10, 12, 13 Symbols #0, 4.. during ABS: Ês1/Noc2 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Ês2/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 2 SNR uncertainty 2 + Cell 2 Fading profile power uncertainty 2) Symbols #1, 2.. during ABS: Ês1/Noc1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc1/Noc2 uncertainty 2) All symbols during non-ABS: Ês1/Noc3 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc3/Noc2 uncertainty 2) Component uncertainties: SNR uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB Noc1/Noc2 and Noc3/Noc2 uncertainty ±0.3 dB |  |  |
| 8.4.1.2.3_E.1 - Propagation Condition EVA5 |  |  | Symbols #0, 4.. during ABS: Ês1/Noc2 ± 0.9 dB Ês2/ Ês1 ± 1.1 dB Ês3/ Ês1 ± 1.1 dB Symbols #1, 2.. during ABS: Ês1/Noc1 ± 1.0 dB All symbols during non-ABS: Ês1/Noc3 ± 1.0 dB |  |  | System uncertainties comprise a number of quantities which are selected according to their applicability in the equations below. Uncertainties are assumed to be uncorrelated, so can be root sum squared. AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Symbols #0, 4.. means 0, 4, 7, 11 Symbols #1, 2.. means 1-3, 5, 6, 8-10, 12, 13 Symbols #0, 4.. during ABS: Ês1/Noc2 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Ês2/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 2 SNR uncertainty 2 + Cell 2 Fading profile power uncertainty 2) Ês3/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 3 SNR uncertainty 2 + Cell 3 Fading profile power uncertainty 2) Symbols #1, 2.. during ABS: Ês1/Noc1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc1/Noc2 uncertainty 2) All symbols during non-ABS: Ês1/Noc3 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc3/Noc2 uncertainty 2) Component uncertainties: SNR uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB Noc1/Noc2 and Noc3/Noc2 uncertainty ±0.3 dB |  |  |
| 8.4.1.2.3_E.2 - Propagation Condition EVA5 |  |  | Same as 8.4.1.2.3_E.1 |  |  | Same as 8.4.1.2.3_E.1 |  |  |
| 8.4.1.2.5 |  |  | Same as 8.2.1.2.5 |  |  | Same as 8.2.1.2.5 |  |  |
| 8.4.1.2.6 |  |  | Same as 8.2.1.2.5 |  |  | Same as 8.2.1.2.5 |  |  |
| 8.4.1.2.7 |  |  | Same as 8.2.1.2.6 |  |  | Same as 8.2.1.2.6 |  |  |
| 8.4.1.2.8 |  |  | Same as 8.2.1.2.6 |  |  | Same as 8.2.1.2.6 |  |  |
| 8.4.2.1 |  |  | ± 0.8 dB |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.2 dB |  |  |
| 8.4.2.2.1 |  |  | ± 1.0 dB |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.4 dB |  |  |
| 8.4.2.2.1_1 |  |  | Same as 8.4.2.2.1 |  |  |  |  |  |
| 8.4.2.2.2 |  |  | ± 1.0 dB |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.4 dB |  |  |
| 8.4.2.2.2_1 |  |  | Same as 8.4.2.2.2 |  |  |  |  |  |
| 8.4.2.2.3_C.1 |  |  | Same as 8.4.1.2.3_C.1 |  |  |  |  |  |
| 8.4.2.2.3_C.2 |  |  | Same as 8.4.1.2.3_C.2 |  |  |  |  |  |
| 8.4.2.2.3_E.1 - Propagation Condition EVA5 |  |  | Symbols #0, 4.. during ABS: Ês1/Noc2 ± 0.9 dB Ês2/ Ês1 ± 1.1 dB Ês3/ Ês1 ± 1.1 dB Symbols #1, 2.. during ABS: Ês1/Noc1 ± 1.0 dB All symbols during non-ABS: Ês1/Noc3 ± 1.0 dB |  |  | System uncertainties comprise a number of quantities which are selected according to their applicability in the equations below. Uncertainties are assumed to be uncorrelated, so can be root sum squared. AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Symbols #0, 4.. means 0, 4, 7, 11 Symbols #1, 2.. means 1-3, 5, 6, 8-10, 12, 13 Symbols #0, 4.. during ABS: Ês1/Noc2 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Ês2/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 2 SNR uncertainty 2 + Cell 2 Fading profile power uncertainty 2) Ês3/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 3 SNR uncertainty 2 + Cell 3 Fading profile power uncertainty 2) Symbols #1, 2.. during ABS: Ês1/Noc1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc1/Noc2 uncertainty 2) All symbols during non-ABS: Ês1/Noc3 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc3/Noc2 uncertainty 2) Component uncertainties: SNR uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB Noc1/Noc2 and Noc3/Noc2 uncertainty ±0.3 dB |  |  |
| 8.4.2.2.3_E.2 - Propagation Condition EVA5 |  |  | Same as 8.4.2.2.3_E.1 |  |  | Same as 8.4.2.2.3_E.1 |  |  |
| 8.4.2.2.5 |  |  | Same as 8.2.1.2.5 |  |  | Same as 8.2.1.2.5 |  |  |
| 8.4.2.2.6 |  |  | Same as 8.2.1.2.5 |  |  | Same as 8.2.1.2.5 |  |  |
| 8.4.2.2.7 |  |  | Same as 8.2.1.2.6 |  |  | Same as 8.2.1.2.6 |  |  |
| 8.4.2.2.8 |  |  | Same as 8.2.1.2.6 |  |  | Same as 8.2.1.2.6 |  |  |
| 8.4.3.1.1 |  |  | Same as 8.2.1.3.1 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.4.3.1.2 |  |  | Same as 8.2.2.3.1 for each CC |  |  | Same as 8.2.2.3.1 Calculation applies for each CC |  |  |
| 8.5.1.1 |  |  | ± 0.9 dB |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.4 dB |  |  |
| 8.5.1.2.1 |  |  | ± 1.1 dB |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.6 dB |  |  |
| 8.5.1.2.1_1 |  |  | Same as 8.5.1.2.1 |  |  |  |  |  |
| 8.5.1.2.2 |  |  | ± 1.0 dB |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.4 dB |  |  |
| 8.5.1.2.2_1 |  |  | Same as 8.5.1.2.2 |  |  |  |  |  |
| 8.5.1.2.3_C.1 |  |  | Symbols #0, 4.. during ABS: Ês1/Noc2 ± 0.9 dB Ês2/ Ês1 ± 1.1 dB Symbols #1, 2.. during ABS: Ês1/Noc1 ± 1.0 dB All symbols during non-ABS: Ês1/Noc3 ± 1.0 dB |  |  | System uncertainties comprise a number of quantities which are selected according to their applicability in the equations below. Uncertainties are assumed to be uncorrelated, so can be root sum squared. AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Symbols #0, 4.. means 0, 4, 7, 11 Symbols #1, 2.. means 1-3, 5, 6, 8-10, 12, 13 Symbols #0, 4.. during ABS: Ês1/Noc2 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Ês2/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 2 SNR uncertainty 2 + Cell 2 Fading profile power uncertainty 2) Symbols #1, 2.. during ABS: Ês1/Noc1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc1/Noc2 uncertainty 2) All symbols during non-ABS: Ês1/Noc3 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc3/Noc2 uncertainty 2) Component uncertainties: SNR uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB Noc1/Noc2 and Noc3/Noc2 uncertainty ±0.3 dB |  |  |
| 8.5.1.2.3_E.1 |  |  | Symbols #0, 4.. during ABS: Ês1/Noc2 ± 0.9 dB Ês2/ Ês1 ± 1.1 dB Ês3/ Ês1 ± 1.1 dB Symbols #1, 2.. during ABS: Ês1/Noc1 ± 1.0 dB All symbols during non-ABS: Ês1/Noc3 ± 1.0 dB |  |  | System uncertainties comprise a number of quantities which are selected according to their applicability in the equations below. Uncertainties are assumed to be uncorrelated, so can be root sum squared. AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Symbols #0, 4.. means 0, 4, 7, 11 Symbols #1, 2.. means 1-3, 5, 6, 8-10, 12, 13 Symbols #0, 4.. during ABS: Ês1/Noc2 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Ês2/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 2 SNR uncertainty 2 + Cell 2 Fading profile power uncertainty 2) Ês3/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 3 SNR uncertainty 2 + Cell 3 Fading profile power uncertainty 2) Symbols #1, 2.. during ABS: Ês1/Noc1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc1/Noc2 uncertainty 2) All symbols during non-ABS: Ês1/Noc3 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc3/Noc2 uncertainty 2) Component uncertainties: SNR uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB Noc1/Noc2 and Noc3/Noc2 uncertainty ±0.3 dB |  |  |
| 8.5.1.2.5 |  |  | Same as 8.2.1.2.5 |  |  | Same as 8.2.1.2.5 |  |  |
| 8.5.1.2.6 |  |  | Same as 8.2.1.2.5 |  |  | Same as 8.2.1.2.5 |  |  |
| 8.5.1.2.7 |  |  | Same as 8.2.1.2.6 |  |  | Same as 8.2.1.2.6 |  |  |
| 8.5.1.2.8 |  |  | Same as 8.2.1.2.6 |  |  | Same as 8.2.1.2.6 |  |  |
| 8.5.2.1 |  |  | ± 0.9 dB |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.4 dB |  |  |
| 8.5.2.2.1 |  |  | ± 1.1 dB |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.6 dB |  |  |
| 8.5.1.2.2_1 |  |  | Same as 8.5.2.2.1 |  |  |  |  |  |
| 8.5.2.2.2 |  |  | ± 1.0 dB |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.4 dB |  |  |
| 8.5.2.2.2_1 |  |  | Same as 8.5.2.2.2 |  |  |  |  |  |
| 8.5.2.2.3_C.1 |  |  | Same as 8.5.1.2.3_C.1 |  |  |  |  |  |
| 8.5.2.2.3_E.1 |  |  | Same as 8.5.1.2.3_E.1 |  |  |  |  |  |
| 8.5.2.2.5 |  |  | Same as 8.2.1.2.5 |  |  | Same as 8.2.1.2.5 |  |  |
| 8.5.2.2.6 |  |  | Same as 8.2.1.2.5 |  |  | Same as 8.2.1.2.5 |  |  |
| 8.5.2.2.7 |  |  | Same as 8.2.1.2.6 |  |  | Same as 8.2.1.2.6 |  |  |
| 8.5.2.2.8 |  |  | Same as 8.2.1.2.6 |  |  | Same as 8.2.1.2.6 |  |  |
| 8.7.1.1 |  |  | Downlink absolute power uncertainty, averaged over BWConfig ±1.0 dB Downlink EVM ≤ 3% |  |  | 3% EVM is equivalent to a Test system downlink SNR of 30.5dB. The noise from the Test system is then sufficiently below that required for the UE to demodulate the signal with the required % success rate. Under these conditions the UE throughput is limited by the Reference measurement channel and the UE capability, and not by the Test system EVM. |  |  |
| 8.7.1.1_1 |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.1.1_2 |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.1.1_A.1 |  |  | Same as 8.7.1.1 for each CC |  |  | Same as 8.7.1.1 Calculation applies for each CC |  |  |
| 8.7.1.1_A.2 |  |  | Same as 8.7.1.1 for each CC |  |  | Same as 8.7.1.1 Calculation applies for each CC |  |  |
| 8.7.1.1_A.4 |  |  | Same as 8.7.1.1 for each CC |  |  | Same as 8.7.1.1 Calculation applies for each CC |  |  |
| 8.7.1.1_A.5 |  |  | Same as 8.7.1.1 for each CC |  |  | Same as 8.7.1.1 Calculation applies for each CC |  |  |
| 8.7.1.1_A.6 |  |  | Same as 8.7.1.1 for each CC |  |  | Same as 8.7.1.1 Calculation applies for each CC |  |  |
| 8.7.1.1_A.7 |  |  | Same as 8.7.1.1 for each CC |  |  | Same as 8.7.1.1 Calculation applies for each CC |  |  |
| 8.7.1.1_H.1 |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.1.1_H.2 |  |  | Same as 8.7.1.1 for each CC |  |  | Same as 8.7.1.1 Calculation applies for each CC |  |  |
| 8.7.1.1_H.3 |  |  | Same as 8.7.1.1 for each CC |  |  | Same as 8.7.1.1 Calculation applies for each CC |  |  |
| 8.7.1.1_H.4 |  |  | Same as 8.7.1.1 for each CC |  |  | Same as 8.7.1.1 Calculation applies for each CC |  |  |
| 8.7.1.1_H.5 |  |  | Same as 8.7.1.1 for each CC |  |  | Same as 8.7.1.1 Calculation applies for each CC |  |  |
| 8.7.1.1_H.6 |  |  | Same as 8.7.1.1 for each CC |  |  | Same as 8.7.1.1 Calculation applies for each CC |  |  |
| 8.7.1.1_H.7 |  |  | Same as 8.7.1.1 for each CC |  |  | Same as 8.7.1.1 Calculation applies for each CC |  |  |
| 8.7.2.1 |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.2.1_1 |  |  | Same as 8.7.2.1 |  |  | Same as 8.7.2.1 |  |  |
| 8.7.2.1_2 |  |  | Same as 8.7.2.1 |  |  | Same as 8.7.2.1 |  |  |
| 8.7.2.1_A.1 |  |  | Same as 8.7.2.1 for each CC |  |  | Same as 8.7.2.1 Calculation applies for each CC |  |  |
| 8.7.2.1_A.2 |  |  | Same as 8.7.2.1 for each CC |  |  | Same as 8.7.2.1 Calculation applies for each CC |  |  |
| 8.7.2.1_A.3 |  |  | Same as 8.7.2.1 for each CC |  |  | Same as 8.7.2.1 Calculation applies for each CC |  |  |
| 8.7.2.1_A.5 |  |  | Same as 8.7.2.1 for each CC |  |  | Same as 8.7.2.1 Calculation applies for each CC |  |  |
| 8.7.2.1_A.6 |  |  | Same as 8.7.2.1 for each CC |  |  | Same as 8.7.2.1 Calculation applies for each CC |  |  |
| 8.7.2.1_A.7 |  |  | Same as 8.7.2.1 for each CC |  |  | Same as 8.7.2.1 Calculation applies for each CC |  |  |
| 8.7.2.1_H.1 |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.2.1_H.1 |  |  | Same as 8.7.2.1 for each CC |  |  | Same as 8.7.2.1 Calculation applies for each CC |  |  |
| 8.7.2.1_H.1 |  |  | Same as 8.7.2.1 for each CC |  |  | Same as 8.7.2.1 Calculation applies for each CC |  |  |
| 8.7.2.1_H.4 |  |  | Same as 8.7.2.1 for each CC |  |  | Same as 8.7.2.1 Calculation applies for each CC |  |  |
| 8.7.2.1_H.5 |  |  | Same as 8.7.2.1 for each CC |  |  | Same as 8.7.2.1 Calculation applies for each CC |  |  |
| 8.7.2.1_H.6 |  |  | Same as 8.7.2.1 for each CC |  |  | Same as 8.7.2.1 Calculation applies for each CC |  |  |
| 8.7.2.1_H.7 |  |  | Same as 8.7.2.1 for each CC |  |  | Same as 8.7.2.1 Calculation applies for each CC |  |  |
| 8.7.3.1 |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.4.1 |  |  | Same as 8.7.3.1 |  |  | Same as 8.7.3.1 |  |  |
| 8.7.5.1.1 TDD FDD CA Sustained data rate performance for FDD PCell (2DL CA) |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.5.1.2 TDD FDD CA Sustained data rate performance for FDD PCell (3DL CA) |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.5.1.3 TDD FDD CA Sustained data rate performance for FDD PCell (4DL CA) |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.5.1.4 TDD FDD CA Sustained data rate performance for FDD PCell (5DL CA) |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.5.1.5 TDD FDD CA Sustained data rate performance for FDD PCell (6DL CA) |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.5.1.6 TDD FDD CA Sustained data rate performance for FDD PCell (7DL CA) |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.5.1_H.1 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (2DL CA) |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.5.1_H.2 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (3DL CA) |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.5.1_H.3 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (4DL CA) |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.5.1_H.4 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (5DL CA) |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.5.1_H.5 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (6DL CA) |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.5.2.1 TDD FDD CA Sustained data rate performance for TDD PCell (2DL CA) |  |  | Same as 8.7.2.1 |  |  | Same as 8.7.2.1 |  |  |
| 8.7.5.2.2 TDD FDD CA Sustained data rate performance for TDD PCell (3DL CA) |  |  | Same as 8.7.2.1 |  |  | Same as 8.7.2.1 |  |  |
| 8.7.5.2.3 TDD FDD CA Sustained data rate performance for TDD PCell (4DL CA) |  |  | Same as 8.7.2.1 |  |  | Same as 8.7.2.1 |  |  |
| 8.7.5.2.4 TDD FDD CA Sustained data rate performance for TDD PCell (5DL CA) |  |  | Same as 8.7.2.1 |  |  | Same as 8.7.2.1 |  |  |
| 8.7.5.2.5 TDD FDD CA Sustained data rate performance for TDD PCell (6DL CA) |  |  | Same as 8.7.2.1 |  |  | Same as 8.7.2.1 |  |  |
| 8.7.5.2.6 TDD FDD CA Sustained data rate performance for TDD PCell (7DL CA) |  |  | Same as 8.7.2.1 |  |  | Same as 8.7.2.1 |  |  |
| 8.7.5.2_H.1 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (2DL CA) |  |  | Same as 8.7.2.1 |  |  | Same as 8.7.2.1 |  |  |
| 8.7.5.2_H.2 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (3DL CA) |  |  | Same as 8.7.2.1 |  |  | Same as 8.7.2.1 |  |  |
| 8.7.5.2_H.3 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (4DL CA) |  |  | Same as 8.7.2.1 |  |  | Same as 8.7.2.1 |  |  |
| 8.7.5.2_H.4 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (5DL CA) |  |  | Same as 8.7.2.1 |  |  | Same as 8.7.2.1 |  |  |
| 8.7.5.2_H.5 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (6DL CA) |  |  | Same as 8.7.2.1 |  |  | Same as 8.7.2.1 |  |  |
| 8.7.6.1 FDD sustained data rate performance for Dual Connectivity 64QAM |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.6.2 FDD sustained data rate performance for Dual Connectivity 256QAM |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.7.1 TDD sustained data rate performance for Dual Connectivity 64QAM |  |  | Same as 8.7.2.1 |  |  | Same as 8.7.2.1 |  |  |
| 8.7.7.2 TDD sustained data rate performance for Dual Connectivity 256QAM |  |  | Same as 8.7.2.1 |  |  | Same as 8.7.2.1 |  |  |
| 8.7.9.2 FDD sustained data rate performance for 4 layer MIMO (single carrier) |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.9.3 FDD sustained data rate performance for 4 layer MIMO (2DL CA) |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.9.4 FDD sustained data rate performance for 4 layer MIMO (3DL CA) |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.9.5 FDD sustained data rate performance for 4 layer MIMO (4DL CA) |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.9.6 FDD sustained data rate performance for 4 layer MIMO (5DL CA) |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.10.2 TDD sustained data rate performance for 4 layer MIMO (single carrier) |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.10.3 TDD sustained data rate performance for 4 layer MIMO (2DL CA) |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.10.4 TDD sustained data rate performance for 4 layer MIMO (3DL CA) |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.10.5 TDD sustained data rate performance for 4 layer MIMO (4DL CA) |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.10.6 TDD sustained data rate performance for 4 layer MIMO (5DL CA) |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.11.2 TDD FDD CA sustained data rate performance for 4 layer MIMO (2DL CA) |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.11.3 TDD FDD CA sustained data rate performance for 4 layer MIMO (3DL CA) |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.11.4 TDD FDD CA sustained data rate performance for 4 layer MIMO (4DL CA) |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.11.5 TDD FDD CA sustained data rate performance for 4 layer MIMO (5DL CA) |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.12.1.2.1 FDD as Pcell with 2DL CA (2RX) |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.12.1.2.2 FDD as Pcell with 2DL CA (4 layer MIMO) |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.12.2.2.1 LAA sustained data rate performance with TDD PCell with 2DL CA (2Rx) |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.7.12.2.2.2 LAA sustained data rate performance with TDD PCell for 4 layer MIMO (2DL CA) |  |  | Same as 8.7.1.1 |  |  | Same as 8.7.1.1 |  |  |
| 8.8.1.1 |  |  | ± 0.9 dB |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.2 dB |  |  |
| 8.8.1.2 |  |  | Same as 8.8.1.1 |  |  | Same as 8.8.1.1 |  |  |
| 8.8.2.1 |  |  | Same as 8.8.1.1 |  |  | Same as 8.8.1.1 |  |  |
| 8.8.2.2 |  |  | Same as 8.8.1.1 |  |  | Same as 8.8.1.1 |  |  |
| 8.8.3.1 |  |  | Same as 8.8.1.1 |  |  | Same as 8.8.1.1 |  |  |
| 8.8.3.2 |  |  | Same as 8.8.1.1 |  |  | Same as 8.8.1.1 |  |  |
| 8.8.4.1 |  |  | Same as 8.2.1.2.5 |  |  | Same as 8.2.1.2.5 |  |  |
| 8.8.4.2 |  |  | Same as 8.2.1.2.5 |  |  | Same as 8.2.1.2.5 |  |  |
| 8.8.5.1 |  |  | Same as 8.2.1.2.5 |  |  | Same as 8.2.1.2.5 |  |  |
| 8.8.6.1 |  |  | Same as 8.2.1.2.5 |  |  | Same as 8.2.1.2.5 |  |  |
| 8.9.1.1.1 |  |  | ± 0.8 dB |  |  | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB |  |  |
| 8.9.1.1.2 |  |  | Same as 8.9.1.1.1 |  |  | Same as 8.9.1.1.1 |  |  |
| 8.9.1.1.3 |  |  | Same as 8.9.1.1.1 |  |  | Same as 8.9.1.1.1 |  |  |
| 8.9.1.1.3_1 |  |  | Same as 8.9.1.1.1 |  |  | Same as 8.9.1.1.,1 |  |  |
| 8.9.1.1.1_1 FDD PDSCH Transmit Diversity 2x1 for UE category 1bis |  |  | ± 0.8 dB |  |  | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB |  |  |
| 8.9.1.1.2_1 FDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x1 for UE Category 1bis |  |  | Same as 8.9.1.1.1_1 |  |  | Same as 8.9.1.1.1_1 |  |  |
| 8.9.1.2.1 |  |  | Same as 8.9.1.1.1 |  |  | Same as 8.9.1.1.1 |  |  |
| 8.9.1.2.1_1 TDD PDSCH Transmit Diversity 2x1 for UE category 1bis |  |  | ± 0.8 dB |  |  | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB |  |  |
| 8.9.1.2.2 |  |  | Same as 8.9.1.1.1 |  |  | Same as 8.9.1.1.1 |  |  |
| 8.9.1.2.3 |  |  | Same as 8.9.1.1.1 |  |  | Same as 8.9.1.1.1 |  |  |
| 8.9.1.2.3_1 |  |  | Same as 8.9.1.1.1 |  |  | Same as 8.9.1.1.1 |  |  |
| 8.9.1.2.2_1 TDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x1 for UE Category 1bis |  |  | Same as 8.9.1.1.1_1 |  |  | Same as 8.9.1.1.1_1 |  |  |
| 8.9.2.1.1 |  |  | ± 0.9 dB |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.4 dB |  |  |
| 8.9.2.1.1_1 |  |  | Same as 8.9.2.1.1 |  |  | Same as 8.9.2.1.1 |  |  |
| 8.9.2.2.1 |  |  | ± 0.9 dB |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.4 dB |  |  |
| 8.9.2.2.1_1 |  |  | Same as 8.9.2.2.1 |  |  | Same as 8.9.2.2.1 |  |  |
| 8.10.1.1.1 |  |  | ± 0.9 dB |  |  | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB |  |  |
| 8.10.1.1.2 |  |  | Same as 8.10.1.1.1 |  |  | Same as 8.10.1.1.1 |  |  |
| 8.10.1.1.3 |  |  | Ês/Noc ± 0.8 dB for each cell |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |  |
| 8.10.1.1.4 |  |  | Same as 8.10.1.1.1 |  |  | Same as 8.10.1.1.1 |  |  |
| 8.10.1.1.5 |  |  | Ês/Noc ± 0.8 dB for each cell |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |  |
| 8.10.1.1.6 |  |  | Same as 8.3.1.2.1_D_1 |  |  | Same as 8.3.1.2.1_D_1 |  |  |
| 8.10.1.1.7 FDD Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports |  |  | Same as 8.2.1.3.1 |  |  | Same as 8.2.1.3.1 |  |  |
| 8.10.1.1.8 FDD Closed-loop spatial multiplexing performance, 4 Layers spatial multiplexing 4 Tx antennas |  |  | Same as 8.2.1.3.1 |  |  | Same as 8.2.1.3.1 |  |  |
| 8.10.1.1.9 FDD 4 Layer Spatial Multiplexing (User-Specific Reference Symbols) |  |  | Same as 8.2.1.3.1 |  |  | Same as 8.2.1.3.1 |  |  |
| 8.10.1.2.1 |  |  | Same as 8.10.1.1.1 |  |  | Same as 8.10.1.1.1 |  |  |
| 8.10.1.2.2 |  |  | Same as 8.10.1.1.1 |  |  | Same as 8.10.1.1.1 |  |  |
| 8.10.1.2.3 |  |  | Ês/Noc ± 0.8 dB for each cell |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for 2 Tx AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |  |
| 8.10.1.2.4 |  |  | Same as 8.10.1.1.1 |  |  | Same as 8.10.1.1.1 |  |  |
| 8.10.1.2.5 |  |  | Ês/Noc ± 0.8 dB for each cell |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for 2 Tx AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |  |
| 8.10.1.2.6 |  |  | Same as 8.3.2.2.1_D_1 |  |  | Same as 8.3.2.2.1_D_1 |  |  |
| 8.10.1.2.7 TDD Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports |  |  | Same as 8.2.2.3.1 |  |  | Same as 8.2.2.3.1 |  |  |
| 8.10.1.2.8 TDD Closed-loop spatial multiplexing performance, 4 Layers spatial multiplexing 4 Tx antennas |  |  | Same as 8.2.2.3.1 |  |  | Same as 8.2.2.3.1 |  |  |
| 8.10.1.2.9 TDD 4 Layer Spatial Multiplexing (User-Specific Reference Symbols) |  |  | Same as 8.2.2.3.1 |  |  | Same as 8.2.2.3.1 |  |  |
| 8.10.2.1.1 FDD PCFICH/PDCCH Single-antenna Port Performance 1x4 |  |  | Same as 8.4.1.1 |  |  |  |  |  |
| 8.10.2.1.2 FDD PCFICH/PDCCH Transmit Diversity 2x4 |  |  | Same as 8.4.1.2.1 |  |  |  |  |  |
| 8.10.2.1.3 FDD PCFICH/PDCCH Transmit Diversity 4x4 |  |  | Same as 8.4.1.2.1 |  |  |  |  |  |
| 8.10.2.2.1 TDD PCFICH/PDCCH Single-antenna Port Performance 1x4 |  |  | Same as 8.4.2.1 |  |  |  |  |  |
| 8.10.2.2.2 TDD PCFICH/PDCCH Transmit Diversity 2x4 |  |  | Same as 8.4.2.2.1 |  |  |  |  |  |
| 8.10.2.2.3 TDD PCFICH/PDCCH Transmit Diversity 4x4 |  |  | Same as 8.4.2.2.1 |  |  |  |  |  |
| 8.10.3.1.1 FDD PHICH Single-antenna Port Performance 1x4 |  |  | Same as 8.5.1.1 |  |  |  |  |  |
| 8.10.3.1.2 FDD PHICH Transmit Diversity 2x4 |  |  | Same as 8.5.1.2.1 |  |  |  |  |  |
| 8.10.3.1.3 FDD PHICH Transmit Diversity 4x4 |  |  | Same as 8.5.1.2.2 |  |  |  |  |  |
| 8.10.3.2.1 TDD PHICH Single-antenna Port Performance 1x4 |  |  | Same as 8.5.2.1 |  |  |  |  |  |
| 8.10.3.2.2 TDD PHICH Transmit Diversity 2x4 |  |  | Same as 8.5.2.2.1 |  |  |  |  |  |
| 8.10.3.2.3 TDD PHICH Transmit Diversity 4x4 |  |  | Same as 8.5.2.2.2 |  |  |  |  |  |
| 8.10.4.1.1 |  |  | ± 0.9 dB |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.2 dB |  |  |
| 8.10.4.1.2 |  |  | Same as 8.10.4.1.1 |  |  | Same as 8.10.4.1.1 |  |  |
| 8.10.4.2.1 |  |  | Same as 8.10.4.1.1 |  |  | Same as 8.10.4.1.1 |  |  |
| 8.10.4.2.2 |  |  | Same as 8.10.4.1.1 |  |  | Same as 8.10.4.1.1 |  |  |
| 8.11.1.1.1 Closed-loop spatial multiplexing performance for UE supporting CE |  |  | ± 0.9 dB |  |  | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for 2x1 spatial multiplexing AWGN flatness and signal flatness ±2.0 dB |  |  |
| 8.11.1.1.2 FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE supporting CE |  |  | Same as 8.11.1.1.1 |  |  | Same as 8.11.1.1.1 |  |  |
| 8.11.1.1.3.1 FDD PDSCH Transmit Diversity 2x1 for UE category M1 |  |  | Same as 8.11.1.1.1 |  |  | Same as 8.11.1.1.1 |  |  |
| 8.11.1.1.3.1_1 FDD PDSCH Transmit Diversity 2x1 for UE category M1 (CEModeB) |  |  | Same as 8.11.1.1.1 |  |  | Same as 8.11.1.1.1 |  |  |
| 8.11.1.1.3.2 FDD and half-duplex FDD PDSCH Transmit Diversity 2x1 for UE category M2 |  |  | Same as 8.11.1.1.1 |  |  | Same as 8.11.1.1.1 |  |  |
| 8.11.1.1.3.2_1 FDD and half-duplex FDD PDSCH Transmit Diversity 2x1 for UE category M2 (CEModeB) |  |  | Same as 8.11.1.1.1 |  |  | Same as 8.11.1.1.1 |  |  |
| 8.11.1.2.1 TDD Closed-loop spatial multiplexing performance in CE Mode A (Cell-Specific Reference Symbols) |  |  | Same as 8.11.1.1.1 |  |  | Same as 8.11.1.1.1 |  |  |
| 8.11.1.2.2 TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 in CE Mode |  |  | Same as 8.11.1.1.1 |  |  | Same as 8.11.1.1.1 |  |  |
| 8.11.1.2.3.1 TDD PDSCH Transmit Diversity for UE category M1 |  |  | Same as 8.11.1.1.1 |  |  | Same as 8.11.1.1.1 |  |  |
| 8.11.1.2.3.1_1 TDD PDSCH Transmit Diversity for UE category M1 (CEModeB) |  |  | Same as 8.11.1.1.1 |  |  | Same as 8.11.1.1.1 |  |  |
| 8.11.1.2.3.2 TDD PDSCH 2 Tx Antenna Port supporting wideband transmission for UE category M2 |  |  | Same as 8.11.1.1.1 |  |  | Same as 8.11.1.1.1 |  |  |
| 8.11.1.2.3.2_1 TDD PDSCH 2 Tx Antenna Port supporting wideband transmission for UE category M2 (CEModeB) |  |  | Same as 8.11.1.1.1 |  |  | Same as 8.11.1.1.1 |  |  |
| 8.11.2.1.1 FDD and HD-FDD demodulation of MPDCCH in CE Mode A |  |  | ± 0.9 dB |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.2 dB |  |  |
| 8.11.2.1.2 FDD and half-duplex FDD demodulation of MPDCCH in CE Mode B |  |  | Same as 8.11.2.1.1 |  |  | Same as 8.11.2.1.1 |  |  |
| 8.11.2.2.1 TDD demodulation of MPDCCH in CE Mode A |  |  | ± 0.9 dB |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.2 dB |  |  |
| 8.11.2.2.2 TDD demodulation of MPDCCH in CE Mode B |  |  | Same as 8.11.2.2.1 |  |  | Same as 8.11.2.2.1 |  |  |
| 8.12.1.1.1 Demodulation of NPDSCH (Cell-Specific Reference Symbols) in In-band mode for category NB1 |  |  | ± 0.9 dB |  |  | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB |  |  |
| 8.12.1.1.2 Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone and Guard-band mode for category NB1 |  |  | ± 0.8 dB |  |  | Same as 8.2.1.1.1 |  |  |
| 8.12.1.1.3 Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone for NB2 |  |  | ± 0.8 dB |  |  | Same as 8.2.1.1.1 |  |  |
| 8.12.2.1.1 Demodulation of NPDCCH single-antenna performance for category NB1 |  |  | ± 0.8 dB |  |  | Same as 8.4.1.1 |  |  |
| 8.12.2.1.2 Demodulation of NPDCCH Transmit diversity performance for category NB1 |  |  | ± 0.9 dB |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.2 dB |  |  |
| 8.13.1.1.1.2 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.1.1.1.3 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.1.1.1.4 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.1.1.1.5 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.1.1.3 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.1.1.4 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.1.2.2 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.1.2.3 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.1.2.4 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.1.2.5 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.1.3.1 |  |  | Same as 8.10.1.1.3 for each CC |  |  | Same as 8.10.1.1.3 Calculation applies for each CC |  |  |
| 8.13.1.4.1 |  |  | Same as 8.10.1.1.5 for each CC |  |  | Same as 8.10.1.1.5 Calculation applies for each CC |  |  |
| 8.13.2.1.1.2 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.2.1.1.3 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.2.1.1.4 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.2.1.1.5 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.2.1.3 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.2.1.4 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.2.2.2 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.2.2.3 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.2.2.4 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.2.2.5 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.2.3.1 |  |  | Same as 8.10.1.2.3 for each CC |  |  | Same as 8.10.1.2.3 Calculation applies for each CC |  |  |
| 8.13.2.4.1 |  |  | Same as 8.10.1.2.5 for each CC |  |  | Same as 8.10.1.2.5 Calculation applies for each CC |  |  |
| 8.13.3.1.1.2 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.3.1.1.3 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.3.1.1.4 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.3.1.1.5 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.3.1.2.2 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.3.1.2.3 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.3.1.2.4 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.3.1.2.5 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.3.2.3 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.3.2.4 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.3.2.5 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.3.2.6 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.3.2.7 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.3.2.8 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.3.2.9 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.3.2.10 |  |  | Same as 8.2.1.4.2 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.3.3.1 |  |  | Ês/Noc ± 0.8 dB for each cell |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |  |
| 8.13.3.3.2 |  |  | Same as 8.13.3.3.1 |  |  | Same as 8.13.3.3.1 |  |  |
| 8.13.3.4.1 |  |  | Ês/Noc ± 0.8 dB for each cell |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |  |
| 8.13.3.4.2 |  |  | Same as 8.13.3.4.1 |  |  | Same as 8.13.3.4.1 |  |  |
| 8.13.3.6.1 |  |  | Same as 8.2.1.3.1 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.3.6.2 |  |  | Same as 8.2.2.3.1 for each CC |  |  | Same as 8.2.2.3.1 Calculation applies for each CC |  |  |
| 8.13.3.7.1 |  |  | Same as 8.2.1.3.1 for each CC |  |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |  |
| 8.13.3.7.2 |  |  | Same as 8.2.2.3.1 for each CC |  |  | Same as 8.2.2.3.1 Calculation applies for each CC |  |  |
| 8.14.1.1.1 - Propagation Condition EVA30 |  |  | ± 0.9 dB |  |  | Same as 8.2.1.3.1 |  |  |
| 8.14.1.1.2 - Propagation Condition EPA5 |  |  | Same as 8.14.1.1.1 |  |  |  |  |  |
| 8.14.1.2.1 - Propagation Condition EVA30 |  |  | Same as 8.14.1.1.1 |  |  |  |  |  |
| 8.14.1.2.2 - Propagation Condition EPA5 |  |  | Same as 8.14.1.1.1 |  |  |  |  |  |
| 8.14.2.1 - Propagation Condition EVA30 - Propagation Condition EPA5 |  |  | ± 1.0 dB |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.4 dB |  |  |
| 8.14.2.2 |  |  | Same as 8.14.2.1 |  |  |  |  |  |
| 10.1 |  |  | ± 0.9 dB |  |  | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB |  |  |
| 10.2 |  |  | ± 0.9 dB |  |  | Same as 10.1 |  |  |
| 14.2 |  |  | ± 0.8 dB |  |  | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB |  |  |
| 14.3 |  |  | ± 0.8 dB |  |  | Same as 14.2 |  |  |
| 14.4 |  |  | ± 0.6 dB |  |  | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |  |
| 14.6 |  |  | ± 0.8 dB |  |  | Same as 14.2 |  |  |
| 14.7 |  |  | ± 0.6 dB |  |  | Same as 14.4 |  |  |
| 14.8 |  |  | Downlink absolute power uncertainty, averaged over BWConfig ±1.0 dB Downlink EVM ≤ 3% |  |  | 3% EVM is equivalent to a Test system downlink SNR of 30.5dB. The noise from the Test system is then sufficiently below that required for the UE to demodulate the signal with the required % success rate. Under these conditions the UE throughput is limited by the Reference measurement channel and the UE capability, and not by the Test system EVM. |  |  |
| 14.9 |  |  | E-UTRA: Downlink absolute power uncertainty, averaged over BWConfig ±1.0 dB Downlink EVM ≤ 3% Sidelink: ± 0.6 dB |  |  | E-UTRA: Same as 8.7.1.1 Sidelink: Same as 14.4 |  |  |
| [Other tests FFS] |  |  |  |  |  |  |  |  |
| In addition, the following Test System uncertainties and related constraints apply: |  |  |  |  |  |  |  |  |
| AWGN Bandwidth |  |  |  |  |  | ≥ 1.08MHz, 2.7MHz, 4.5MHz, 9MHz, 13.5MHz, 18MHz; NRB x 180kHz according to BWConfig |  |  |
| AWGN absolute power uncertainty, averaged over BWConfig Note 4 |  |  |  |  |  | ±3 dB |  |  |
| AWGN flatness and signal flatness, max deviation for any Resource Block, relative to average over BWConfig |  |  |  |  |  | ±2 dB |  |  |
| AWGN peak to average ratio |  |  |  |  |  | ≥10 dB @0.001% |  |  |
| Signal-to noise ratio uncertainty, averaged over downlink transmission Bandwidth |  |  |  |  |  | ±0.3 dB (includes uncertainty in precoding applied by the test system, where applicable) |  |  |
| Signal-to noise ratio variation for any resource block, relative to average over downlink transmission Bandwidth |  |  |  |  |  | ±0.5 dB |  |  |
| Noc2 absolute power uncertainty, averaged over BWConfig Note 5 |  |  |  |  |  | ±3 dB |  |  |
| Noc1 / Noc2 ratio uncertainty, averaged over BWConfig Note 5 |  |  |  |  |  | ±0.3 dB |  |  |
| Noc3 / Noc2 ratio uncertainty, averaged over BWConfig Note 5 |  |  |  |  |  | ±0.3 dB |  |  |
| Es / Noc2 ratio (SNR) uncertainty, averaged over downlink transmission Bandwidth Note 5 |  |  |  |  |  | ±0.3 dB (includes uncertainty in precoding applied by the test system, where applicable) |  |  |
| Fading profile power uncertainty |  |  |  |  |  | Test-specific |  |  |
| Fading profile delay uncertainty, relative to frame timing |  |  |  |  |  | ±5 ns (excludes absolute errors related to baseband timing) |  |  |
| CA, DC and LAA performance requirements only: Relative frequency error between carriers |  |  |  |  |  | 30Hz, measured over a 1ms period, and maximum carrier spacing 80MHz |  |  |
| enhanced performance requirement Type B only: Frequency error on the frequency offset to cell 1 |  |  |  |  |  | 30Hz, measured over a 1ms period |  |  |
| Note 1: Only the overall stimulus error is considered here. The effect of errors in the throughput measurements due to finite test duration is not considered. Note 2: The AWGN or N ocx parameters apply to all test cases except 8.7.1 and 8.7.2. The fading parameters apply to test cases using fading Note 3: In CA, DC and LAA test cases using multiple component carriers (CCs), the uncertainties and related constraints apply for each CC. Note 4: Applies for test cases which specify Noc, a single value that remains constant with time. Note 5: Applies for test cases which specify Noc1, Noc2 and Noc3, that are symbol or subframe specific. |  |  |  |  |  |  |  |  |

## F.1.5	Measurement of Channel State Information reporting

Table F.1.5-1: Maximum Test System Uncertainty for Channel State Information reporting

| Subclause |  |  | Maximum Test System Uncertainty1 |  |  | Derivation of Test System Uncertainty |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9.2.1.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 |  |  | ± 0.3 dB |  |  | Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect AWGN absolute power uncertainty ±3.0 dB not expected to have any significant effect |  |  |
| 9.2.1.2 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 |  |  | ± 0.3 dB |  |  | Same as 9.2.1.1 |  |  |
| 9.2.1.3_C.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 for eICIC (non-MBSFN ABS) |  |  | Noc2 Noc1 / Noc2 Noc3 / Noc2 Cell 1 Es / Noc2 Cell 2 Es / Noc2 |  |  | Noc2, Noc ratios and Es / Noc2 ratios are generic values as specified at end of this table |  |  |
| 9.2.1.4_C.1 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 for eICIC (non-MBSFN ABS) |  |  | Noc2 Noc1 / Noc2 Noc3 / Noc2 Cell 1 Es / Noc2 Cell 2 Es / Noc2 |  |  | Noc2, Noc ratios and Es / Noc2 ratios are generic values as specified at end of this table |  |  |
| 9.2.1.5_E.1 FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for feICIC (non-MBSFN ABS) |  |  | Noc2 Noc1 / Noc2 Noc3 / Noc2 Cell 1 Es / Noc2 Cell 2 Es / Noc2 Cell 3 Es / Noc2 |  |  | Noc2, Noc ratios and Es / Noc2 ratios are generic values as specified at end of this table |  |  |
| 9.2.1.6_E.1 TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for feICIC (non-MBSFN ABS) |  |  | Noc2 Noc1 / Noc2 Noc3 / Noc2 Cell 1 Es / Noc2 Cell 2 Es / Noc2 Cell 3 Es / Noc2 |  |  | Noc2, Noc ratios and Es / Noc2 ratios are generic values as specified at end of this table |  |  |
| 9.2.1.7 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 for 256QAM in DL |  |  | ± 0.3 dB |  |  | Same as 9.2.1.1 |  |  |
| 9.2.1.8 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 for 256QAM in DL |  |  | ± 0.3 dB |  |  | Same as 9.2.1.1 |  |  |
| 9.2.2.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-1 |  |  | ± 0.3 dB |  |  | Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect AWGN absolute power uncertainty ±3.0 dB not expected to have any significant effect |  |  |
| 9.2.2.2 TDD CQI Reporting under AWGN conditions – PUCCH 1-1 |  |  | ± 0.3 dB |  |  | Same as 9.2.1.1 |  |  |
| 9.2.3.1_D FDD CQI Reporting under AWGN conditions – PUCCH 1-1 for eDL-MIMO |  |  | ± 0.3 dB |  |  | Same as 9.2.2.1 |  |  |
| 9.2.3.2_D TDD CQI Reporting under AWGN conditions – PUCCH 1-1 for eDL-MIMO |  |  | ± 0.3 dB |  |  | Same as 9.2.2.1 |  |  |
| 9.2.3.1A FDD CQI Reporting under AWGN conditions – PUCCH 1-1 for FD-MIMO (With channelMeasRestriction configured) |  |  | ± 0.3 dB |  |  | Same as 9.2.2.1 |  |  |
| 9.2.3.2A TDD CQI Reporting under AWGN conditions – PUCCH 1-1 for FD-MIMO (With channelMeasRestriction configured) |  |  | ± 0.3 dB |  |  | Same as 9.2.2.1 |  |  |
| 9.2.4.1A FDD CQI Reporting under AWGN conditions - Single CSI Process With interferenceMeasRestriction configured |  |  | ± 0.3 dB |  |  | Same as 9.2.2.1 |  |  |
| 9.2.4.1_F FDD CQI Reporting under AWGN conditions - Single CSI Process for CoMP |  |  | ± 0.3 dB |  |  | Same as 9.2.2.1 |  |  |
| 9.2.4.2A TDD CQI Reporting under AWGN conditions - Single CSI Process With interferenceMeasRestriction configured |  |  | ± 0.3 dB |  |  | Same as 9.2.2.1 |  |  |
| 9.2.4.2_F TDD CQI Reporting under AWGN conditions - Single CSI Process for CoMP |  |  | ± 0.3 dB |  |  | Same as 9.2.2.1 |  |  |
| 9.2.5 TDD CQI Reporting under AWGN conditions – PUCCH 1-1 (when csi-SubframeSet – r12 and EIMTA-MainConfigServCell-r12 are configured) |  |  | ± 0.3 dB |  |  | Same as 9.2.1.1 |  |  |
| 9.2.6.1 LAA CQI Reporting under AWGN Conditions with Frame Structure Type 3 with FDD as Pcell (PUSCH 3-0) |  |  | ± 0.3 dB |  |  | Same as 9.2.2.1 |  |  |
| 9.2.6.2 LAA CQI Reporting under AWGN Conditions with Frame Structure Type 3 with TDD as Pcell (PUSCH 3-0) |  |  | ± 0.3 dB |  |  | Same as 9.2.2.1 |  |  |
| 9.2.7.1 LAA CQI Reporting under AWGN Conditions with Frame Structure Type 3 with FDD as Pcell (PUSCH 3-1) |  |  | ± 0.3 dB |  |  | Same as 9.2.1.1 |  |  |
| 9.2.7.2 LAA CQI Reporting under AWGN Conditions with Frame Structure Type 3 with TDD as Pcell (PUSCH 3-1) |  |  | ± 0.3 dB |  |  | Same as 9.2.1.1 |  |  |
| 9.3.1.1.1 FDD CQI Reporting under fading conditions – PUSCH 3-0 |  |  | ± 0.6 dB |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty ±0.3 dB 2. Fading profile power uncertainty ±0.5 dB Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect AWGN absolute power uncertainty ±3.0 dB not expected to have any significant effect |  |  |
| 9.3.1.1.2 TDD CQI Reporting under fading conditions – PUSCH 3-0 |  |  | ± 0.6 dB |  |  | Same as 9.3.1.1.1 |  |  |
| 9.3.1.1.5 TDD CQI Reporting under fading conditions – PUCCH 3-0 (when csi-SubframeSet – r12 is configured) |  |  | ± 0.6 dB |  |  | Same as 9.3.1.1.1 |  |  |
| 9.3.1.2.1_D FDD CQI Reporting under fading conditions – PUSCH 3-1 for eDL-MIMO |  |  | ± 0.6 dB |  |  | Same as 9.3.1.1.1 |  |  |
| 9.3.1.2.2_D TDD CQI Reporting under fading conditions – PUSCH 3-1 for eDL-MIMO |  |  | ± 0.6 dB |  |  | Same as 9.3.1.1.1 |  |  |
| 9.3.1.2.3 FDD CQI Reporting under fading conditions – PUSCH 3-1 for 256QAM in DL |  |  | ± 0.6 dB |  |  | Same as 9.3.1.1.1 |  |  |
| 9.3.1.2.4 TDD CQI Reporting under fading conditions – PUSCH 3-1 for 256QAM in DL |  |  | ± 0.6 dB |  |  | Same as 9.3.1.1.1 |  |  |
| 9.3.1.2.6 TDD CQI Reporting under fading conditions – PUCCH 3-1 (when csi-SubframeSet – r12 is configured with one CSI process) |  |  | ± 0.6 dB |  |  | Same as 9.3.1.1.1 |  |  |
| 9.3.1.3.1_E.1 FDD CQI Reporting under fading conditions - PUSCH 3-0 for feICIC (non-MBSFN ABS) |  |  | Noc2 Noc1 / Noc2 Noc3 / Noc2 Cell 1 Es / Noc2 Cell 2 Es / Noc2 Cell 3 Es / Noc2 Fading profile power uncertainty ±0.5 dB |  |  | Noc2, Noc ratios and Es / Noc2 ratios are generic values as specified at end of this table |  |  |
| 9.3.1.3.2_E.1 TDD CQI Reporting under fading conditions - PUSCH 3-0 for feICIC (non-MBSFN ABS) |  |  | Noc2 Noc1 / Noc2 Noc3 / Noc2 Cell 1 Es / Noc2 Cell 2 Es / Noc2 Cell 3 Es / Noc2 Fading profile power uncertainty ±0.5 dB |  |  | Noc2, Noc ratios and Es / Noc2 ratios are generic values as specified at end of this table |  |  |
| 9.3.2.1.1 FDD CQI Reporting under fading conditions – PUCCH 1-0 |  |  | ± 0.6 dB |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty ±0.3 dB 2. Fading profile power uncertainty ±0.5 dB Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect AWGN absolute power uncertainty ±3.0 dB not expected to have any significant effect |  |  |
| 9.3.2.1.1_1 FDD CQI Reporting under fading conditions – PUCCH 1-0 |  |  | ± 0.6 dB |  |  | Same as 9.3.2.1.1 |  |  |
| 9.3.2.1.2 TDD CQI Reporting under fading conditions – PUCCH 1-0 |  |  | ± 0.6 dB |  |  | Same as 9.3.2.1.1 |  |  |
| 9.3.2.1.2_1 TDD CQI Reporting under fading conditions – PUCCH 1-0 |  |  | ± 0.6 dB |  |  | Same as 9.3.2.1.1 |  |  |
| 9.3.2.2.1_D FDD CQI Reporting under fading conditions – PUCCH 1-1 for eDL-MIMO |  |  | ± 0.6 dB |  |  | Same as 9.3.2.1.1 |  |  |
| 9.3.2.2.2_D TDD CQI Reporting under fading conditions – PUCCH 1-1 for eDL-MIMO |  |  | ± 0.6 dB |  |  | Same as 9.3.2.1.1 |  |  |
| 9.3.3.1.1 FDD CQI Reporting under fading conditions and frequency-selective interference – PUSCH 3-0 |  |  | ± 1.2 dB |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Ior/Iot ratio uncertainty ±1.0 dB 2. Fading profile power uncertainty ±0.5 dB Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Ior/Iot ratio uncertainty 2 + Fading profile power uncertainty 2) Ior absolute power uncertainty ±3.0 dB not expected to have any significant effect |  |  |
| 9.3.3.1.2 TDD CQI Reporting under fading conditions and frequency-selective interference – PUSCH 3-0 |  |  | ± 1.2 dB |  |  | Same as 9.3.3.1.1 |  |  |
| 9.3.4.1.1 FDD CQI Reporting under fading conditions – PUSCH 2-0 |  |  | ± 0.6 dB |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty ±0.3 dB 2. Fading profile power uncertainty ±0.5 dB Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect AWGN absolute power uncertainty ±3.0 dB not expected to have any significant effect |  |  |
| 9.3.4.1.2 TDD CQI Reporting under fading conditions – PUSCH 2-0 |  |  | ± 0.6 dB |  |  | Same as 9.3.4.1.1 |  |  |
| 9.3.4.2.1 FDD CQI Reporting under fading conditions – PUCCH 2-0 |  |  | ± 0.6 dB |  |  | Same as 9.3.4.1.1 |  |  |
| 9.3.4.2.2 TDD CQI Reporting under fading conditions – PUCCH 2-0 |  |  | ± 0.6 dB |  |  | Same as 9.3.4.1.1 |  |  |
| 9.3.5.1.1 FDD CQI Reporting under fading conditions - PUCCH 1-0 - Enhanced Performance Requirement Type A |  |  | Ês1/Noc ± 0.6 dB Ês2/Noc ± 0.3 dB |  |  | Overall system uncertainty for fading conditions on Cell 1 comprises two quantities: 1. Signal-to-noise ratio uncertainty ±0.3 dB 2. Fading profile power uncertainty ±0.5 dB for single Tx Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect AWGN absolute power uncertainty ±3.0 dB not expected to have any significant effect |  |  |
| 9.3.5.1.2 TDD CQI Reporting under fading conditions - PUCCH 1-0 - Enhanced Performance Requirement Type A |  |  | Same as 9.3.5.1.1 |  |  | Same as 9.3.5.1.1 |  |  |
| 9.3.5.2.1 FDD CQI Reporting under fading conditions - PUCCH 1-1 - Enhanced Performance Requirement Type A |  |  | Ês1/Noc ± 0.8 dB Ês2/Noc ± 0.3 dB |  |  | Overall system uncertainty for fading conditions on Cell 1 comprises two quantities: 1. Signal-to-noise ratio uncertainty ±0.3 dB 2. Fading profile power uncertainty ±0.5 dB for two Tx antennas. Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect AWGN absolute power uncertainty ±3.0 dB not expected to have any significant effect |  |  |
| 9.3.5.2.2 TDD CQI Reporting under fading conditions - PUCCH 1-1 - Enhanced Performance Requirement Type A |  |  | Same as 9.3.5.2.1 |  |  | Same as 9.3.5.2.1 |  |  |
| 9.3.6.1_F.1 FDD CQI Reporting under fading conditions Single CSI processes for CoMP |  |  | Ês/Noc ± 0.8 dB |  |  | Overall system uncertainty for fading conditions on TP1 and TP2 comprises two quantities: 1. Signal-to-noise ratio uncertainty ±0.3 dB 2. Fading profile power uncertainty ±0.7 dB for multiple Tx Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect AWGN absolute power uncertainty ±3.0 dB not expected to have any significant effect |  |  |
| 9.3.6.1_F.2 FDD CQI Reporting under fading conditions Three CSI processes for CoMP |  |  | Same as 9.3.6.1_F.1 |  |  | Same as 9.3.6.1_F.1 |  |  |
| 9.3.6.1_F.3 FDD CQI Reporting under fading conditions Four CSI processes for CoMP |  |  | Same as 9.3.6.1_F.1 |  |  | Same as 9.3.6.1_F.1 |  |  |
| 9.3.6.2_F.1 TDD CQI Reporting under fading conditions Single CSI processes for CoMP |  |  | Ês/Noc ± 0.8 dB |  |  | Overall system uncertainty for fading conditions on TP1 and TP2 comprises two quantities: 1. Signal-to-noise ratio uncertainty ±0.3 dB 2. Fading profile power uncertainty ±0.7 dB for multiple Tx Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect AWGN absolute power uncertainty ±3.0 dB not expected to have any significant effect |  |  |
| 9.3.6.2_F.2 TDD CQI Reporting under fading conditions Three CSI processes for CoMP |  |  | Same as 9.3.6.2_F.1 |  |  | Same as 9.3.6.2_F.1 |  |  |
| 9.3.6.2_F.3 TDD CQI Reporting under fading conditions Four CSI processes for CoMP |  |  | Same as 9.3.6.2_F.1 |  |  | Same as 9.3.6.2_F.1 |  |  |
| 9.3.7.1 FDD CQI Reporting under fading conditions - PUSCH 3-2 for eDL MIMO Enhancement |  |  | ± 0.8 dB |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty ±0.3 dB 2. Fading profile power uncertainty ±0.7 dB for multiple Tx Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect AWGN absolute power uncertainty ±3.0 dB not expected to have any significant effect |  |  |
| 9.3.7.2 TDD CQI Reporting under fading conditions - PUSCH 3-2 for eDL MIMO Enhancement |  |  | ± 0.8 dB |  |  | Same as 9.3.7.1 |  |  |
| 9.3.8.1.1 FDD CQI Reporting under fading conditions - PUCCH 1-1 (Cell Specific Reference Symbols) TM4 - Enhanced Performance Receiver Type B |  |  | Ês/Noc ± 0.8 dB for each cell |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect AWGN absolute power uncertainty ±3.0 dB not expected to have any significant effect |  |  |
| 9.3.8.1.2 TDD CQI Reporting under fading conditions - PUCCH 1-1 (Cell Specific Reference Symbols) TM4 - Enhanced Performance Receiver Type B |  |  | Same as 9.3.8.1.1 |  |  | Same as 9.3.8.1.1 |  |  |
| 9.3.8.2.1 FDD CQI Reporting under fading conditions - PUCCH 1-1 (CSI Reference Symbol) TM9 - Enhanced Receiver Type B |  |  | Ês/Noc ± 0.8 dB for each cell |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect AWGN absolute power uncertainty ±3.0 dB not expected to have any significant effect |  |  |
| 9.3.8.2.2 TDD CQI Reporting under fading conditions - PUCCH 1-1 (CSI Reference Symbol) TM9 - Enhanced Receiver Type B |  |  | Same as 9.3.8.2.1 |  |  | Same as 9.3.8.2.1 |  |  |
| 9.3.8.3.1 FDD CQI Reporting under fading conditions - PUCCH 1-1 (CSI Reference Symbol) TM10 with TM9 interference - Enhanced Receiver Type B |  |  | Ês/Noc ± 0.8 dB for each cell |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect AWGN absolute power uncertainty ±3.0 dB not expected to have any significant effect |  |  |
| 9.3.8.3.2 TDD CQI Reporting under fading conditions - PUCCH 1-1 (CSI Reference Symbol) TM10 with TM9 interference - Enhanced Receiver Type B |  |  | Same as 9.3.8.3.1 |  |  | Same as 9.3.8.3.1 |  |  |
| 9.4.1.1.1 FDD PMI Reporting – PUSCH 3-1 (Single PMI) |  |  | ± 0.6 dB |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty ±0.3 dB 2. Fading profile power uncertainty ±0.5 dB Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect AWGN absolute power uncertainty ±3.0 dB not expected to have any significant effect |  |  |
| 9.4.1.1.2 TDD PMI Reporting – PUSCH 3-1 (Single PMI) |  |  | ± 0.6 dB |  |  | Same as 9.4.1.1.1 |  |  |
| 9.4.1.2.1 FDD PMI Reporting – PUCCH 2-1 (Single PMI) |  |  | ± 0.6 dB |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty ±0.3 dB 2. Fading profile power uncertainty ±0.5 dB Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect AWGN absolute power uncertainty ±3.0 dB not expected to have any significant effect |  |  |
| 9.4.1.2.2 TDD PMI Reporting – PUCCH 2-1 (Single PMI) |  |  | ± 0.6 dB |  |  | Same as 9.4.1.2.1 |  |  |
| 9.4.1.3.1_D FDD Reporting of PMI – PUSCH 3-1 (Single PMI) for eDL-MIMO |  |  | ± 0.6 dB |  |  | Same as 9.4.1.1.1 |  |  |
| 9.4.1.3.2_D TDD Reporting of PMI – PUSCH 3-1 (Single PMI) for eDL-MIMO |  |  | ± 0.6 dB |  |  | Same as 9.4.1.1.1 |  |  |
| 9.4.1.3.3 FDD PMI Reporting with 12Tx Class A codebook – PUSCH 3-1 (Single PMI) for FD-MIMO |  |  | ± 0.8 dB |  |  | Same as 9.4.1.4.1 |  |  |
| 9.4.1.3.4 TDD PMI Reporting with 12Tx Class A codebook – PUSCH 3-1 (Single PMI) for FD-MIMO |  |  | ± 0.8 dB |  |  | Same as 9.4.1.4.1 |  |  |
| 9.4.1.4.1 FDD PMI Reporting with 4Tx enhanced codebook – PUCCH 1-1 (Single PMI) for eDL MIMO Enhancement |  |  | ± 0.8 dB |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty ±0.3 dB 2. Fading profile power uncertainty ±0.7 dB for multiple Tx Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect AWGN absolute power uncertainty ±3.0 dB not expected to have any significant effect |  |  |
| 9.4.1.4.2 TDD PMI Reporting with 4Tx enhanced codebook – PUCCH 1-1 (Single PMI) for eDL MIMO Enhancement |  |  | ± 0.8 dB |  |  | Same as 9.4.1.4.1 |  |  |
| 9.4.1.4.3 FDD PMI Reporting with Class B alternative codebook – PUCCH 1-1 for FD-MIMO |  |  | ± 0.8 dB |  |  | Same as 9.4.1.4.1 |  |  |
| 9.4.1.4.4 TDD PMI Reporting with Class B alternative codebook – PUCCH 1-1 for FD-MIMO |  |  | ± 0.8 dB |  |  | Same as 9.4.1.4.1 |  |  |
| 9.4.2.1.1 FDD PMI Reporting – PUSCH 1-2 (Multiple PMI) |  |  | ± 0.6 dB |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty ±0.3 dB 2. Fading profile power uncertainty ±0.5 dB Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect AWGN absolute power uncertainty ±3.0 dB not expected to have any significant effect |  |  |
| 9.4.2.1.1_1 FDD PMI Reporting – PUSCH 1-2 (Multiple PMI) |  |  | ± 0.6 dB |  |  | Same as 9.4.2.1.1 |  |  |
| 9.4.2.1.2 TDD PMI Reporting – PUSCH 1-2 (Multiple PMI) |  |  | ± 0.6 dB |  |  | Same as 9.4.2.1.1 |  |  |
| 9.4.2.1.2_1 TDD PMI Reporting – PUSCH 1-2 (Multiple PMI) |  |  | ± 0.6 dB |  |  | Same as 9.4.2.1.1 |  |  |
| 9.4.2.2.1 FDD PMI Reporting – PUSCH 2-2 (Multiple PMI) |  |  | ± 0.6 dB |  |  | Same as 9.4.2.1.1 |  |  |
| 9.4.2.2.2 TDD PMI Reporting – PUSCH 2-2 (Multiple PMI) |  |  | ± 0.6 dB |  |  | Same as 9.4.2.1.1 |  |  |
| 9.4.2.3.1_D FDD PMI Reporting – PUSCH 1-2 (Multiple PMI) for eDL-MIMO |  |  | ± 0.6 dB |  |  | Same as 9.4.2.1.1 |  |  |
| 9.4.2.3.2_D TDD PMI Reporting – PUSCH 1-2 (Multiple PMI) for eDL-MIMO |  |  | ± 0.6 dB |  |  | Same as 9.4.2.1.1 |  |  |
| 9.4.2.3.3 TDD PMI Reporting with 4Tx enhanced codebook - PUSCH 1-2 (Multiple PMI) for eDL-MIMO Enhancement |  |  | ± 0.8 dB |  |  | Same as 9.4.1.4.1 |  |  |
| 9.4.2.3.4 TDD PMI Reporting with 4Tx enhanced codebook - PUSCH 1-2 (Multiple PMI) for eDL-MIMO Enhancement |  |  | ± 0.8 dB |  |  | Same as 9.4.1.4.1 |  |  |
| 9.4.2.3.5 FDD PMI Reporting with Class A 16Tx codebook - PUSCH 1-2 (Multiple PMI) for FD-MIMO |  |  | ± 0.8 dB |  |  | Same as 9.4.1.4.1 |  |  |
| 9.4.2.3.6 TDD PMI Reporting with Class A 16Tx codebook - PUSCH 1-2 (Multiple PMI) for FD-MIMO |  |  | ± 0.8 dB |  |  | Same as 9.4.1.4.1 |  |  |
| 9.5.1.1 FDD RI Reporting– PUCCH 1-1 |  |  | ± 0.8 dB |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty ±0.3 dB 2. Fading profile power uncertainty ±0.7 dB for MIMO Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect AWGN absolute power uncertainty ±3.0 dB not expected to have any significant effect |  |  |
| 9.5.1.1_1 FDD RI Reporting– PUCCH 1-1 (Release 10) |  |  | ± 0.8 dB |  |  | Same as 9.5.1.1 |  |  |
| 9.5.1.1_2 FDD RI Reporting– PUCCH 1-1 (Release 11) |  |  | ± 0.8 dB |  |  | Same as 9.5.1.1 |  |  |
| 9.5.1.2 TDD RI Reporting– PUSCH 3-1 |  |  | ± 0.8 dB |  |  | Same as 9.5.1.1 |  |  |
| 9.5.1.2_1 TDD RI Reporting– PUSCH 3-1 (Release 10) |  |  | ± 0.8 dB |  |  | Same as 9.5.1.1 |  |  |
| 9.5.1.2_2 TDD RI Reporting– PUSCH 3-1 (Release 11) |  |  | ± 0.8 dB |  |  | Same as 9.5.1.1 |  |  |
| 9.5.2.1_D FDD RI Reporting – PUCCH 1-1 for eDL-MIMO |  |  | ± 0.8 dB |  |  | Same as 9.5.1.1 |  |  |
| 9.5.2.2_D TDD RI Reporting – PUCCH 1-1 for eDL-MIMO |  |  | ± 0.8 dB |  |  | Same as 9.5.1.1 |  |  |
| 9.5.3.1_C.1 FDD RI Reporting – PUCCH 1-0 for eICIC (non-MBSFN ABS) |  |  | Noc2 Noc1 / Noc2 Noc3 / Noc2 Cell 1 Es / Noc2 Cell 2 Es / Noc2 Fading profile power uncertainty ±0.7 dB for MIMO |  |  | Noc2, Noc ratios and Es / Noc2 ratios are generic values as specified at end of this table |  |  |
| 9.5.3.2_C.1 TDD RI Reporting – PUCCH 1-0 for eICIC (non-MBSFN ABS) |  |  | Noc2 Noc1 / Noc2 Noc3 / Noc2 Cell 1 Es / Noc2 Cell 2 Es / Noc2 Fading profile power uncertainty ±0.7 dB for MIMO |  |  | Noc2, Noc ratios and Es / Noc2 ratios are generic values as specified at end of this table |  |  |
| 9.5.4.1_E.1 FDD RI Reporting - PUCCH 1-0 for feICIC (non-MBSFN ABS) |  |  | Noc2 Noc1 / Noc2 Noc3 / Noc2 Cell 1 Es / Noc2 Cell 2 Es / Noc2 Cell 3 Es / Noc2 Fading profile power uncertainty ±0.7 dB for MIMO |  |  | Noc2, Noc ratios and Es / Noc2 ratios are generic values as specified at end of this table |  |  |
| 9.5.4.2_E.1 TDD RI Reporting - PUCCH 1-0 for feICIC (non-MBSFN ABS) |  |  | Noc2 Noc1 / Noc2 Noc3 / Noc2 Cell 1 Es / Noc2 Cell 2 Es / Noc2 Cell 3 Es / Noc2 Fading profile power uncertainty ±0.7 dB for MIMO |  |  | Noc2, Noc ratios and Es / Noc2 ratios are generic values as specified at end of this table |  |  |
| 9.5.5.1_F.1 FDD RI Reporting with Single CSI process for CoMP |  |  | ± 0.8 dB for each TP |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty ±0.3 dB 2. Fading profile power uncertainty ±0.7 dB for MIMO Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect AWGN absolute power uncertainty ±3.0 dB not expected to have any significant effect |  |  |
| 9.5.5.1_F.2 FDD RI Reporting with Multiple CSI processes for CoMP |  |  | ± 0.8 dB for each TP |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty ±0.3 dB 2. Fading profile power uncertainty ±0.7 dB for MIMO Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect AWGN absolute power uncertainty ±3.0 dB not expected to have any significant effect |  |  |
| 9.5.5.2_F.1 TDD RI Reporting with Single CSI process for CoMP |  |  | ± 0.8 dB for each TP |  |  | Same as 9.5.5.1_F.1 |  |  |
| 9.5.5.2_F.2 TDD RI Reporting with Multiple CSI processes for CoMP |  |  | ± 0.8 dB for each TP |  |  | Same as 9.5.5.1_F.2 |  |  |
| 9.6.1.1_A.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 for CA (2DL CA) |  |  | ± 0.3 dB for each CC |  |  | Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect AWGN absolute power uncertainty ±3.0 dB not expected to have any significant effect |  |  |
| 9.6.1.1_A.2 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 (3DL CA) |  |  | Same as 9.6.1.1_A.1 |  |  | Same as 9.6.1.1_A.1 |  |  |
| 9.6.1.1_A.4 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 (5DL CA) |  |  | Same as 9.6.1.1_A.1 |  |  | Same as 9.6.1.1_A.1 |  |  |
| 9.6.1.1_A.5 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 (6DL CA) |  |  | Same as 9.6.1.1_A.1 |  |  | Same as 9.6.1.1_A.1 |  |  |
| 9.6.1.1_A.6 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 (7DL CA) |  |  | Same as 9.6.1.1_A.1 |  |  | Same as 9.6.1.1_A.1 |  |  |
| 9.6.1.2_A.1 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 for 2DL CA |  |  | Same as 9.6.1.1_A.1 |  |  | Same as 9.6.1.1_A.1 |  |  |
| 9.6.1.2_A.2 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 (3DL CA) |  |  | Same as 9.6.1.1_A.1 |  |  | Same as 9.6.1.1_A.1 |  |  |
| 9.6.1.2_A.3 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 (4DL CA) |  |  | Same as 9.6.1.1_A.1 |  |  | Same as 9.6.1.1_A.1 |  |  |
| 9.6.1.2_A.4 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 (5DL CA) |  |  | Same as 9.6.1.1_A.1 |  |  | Same as 9.6.1.1_A.1 |  |  |
| 9.6.1.2_A.5 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 (6DL CA) |  |  | Same as 9.6.1.1_A.1 |  |  | Same as 9.6.1.1_A.1 |  |  |
| 9.6.1.2_A.6 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 (7DL CA) |  |  | Same as 9.6.1.1_A.1 |  |  | Same as 9.6.1.1_A.1 |  |  |
| 9.6.1.3.1 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for FDD PCell (2DL CA) |  |  | Same as 9.6.1.1_A.1 |  |  | Same as 9.6.1.1_A.1 |  |  |
| 9.6.1.3.2 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for FDD PCell (3DL CA) |  |  | Same as 9.6.1.1_A.1 |  |  | Same as 9.6.1.1_A.1 |  |  |
| 9.6.1.3.3 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for FDD PCell (4DL CA) |  |  | Same as 9.6.1.1_A.1 |  |  | Same as 9.6.1.1_A.1 |  |  |
| 9.6.1.3.4 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for FDD PCell (5DL CA) |  |  | Same as 9.6.1.1_A.1 |  |  | Same as 9.6.1.1_A.1 |  |  |
| 9.6.1.4.1 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for TDD PCell (2DL CA) |  |  | Same as 9.6.1.1_A.1 |  |  | Same as 9.6.1.1_A.1 |  |  |
| 9.6.1.4.2 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for TDD PCell (3DL CA) |  |  | Same as 9.6.1.1_A.1 |  |  | Same as 9.6.1.1_A.1 |  |  |
| 9.6.1.4.3 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for TDD PCell (4DL CA) |  |  | Same as 9.6.1.1_A.1 |  |  | Same as 9.6.1.1_A.1 |  |  |
| 9.6.1.4.4 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for TDD PCell (5DL CA) |  |  | Same as 9.6.1.1_A.1 |  |  | Same as 9.6.1.1_A.1 |  |  |
| 9.7.1.1 FDD and Half duplex FDD CQI reporting definition under AWGN conditions for UE category 0 |  |  | ± 0.3 dB |  |  | Same as 9.2.1.1 |  |  |
| 9.7.1.2 TDD CQI reporting definition under AWGN conditions for UE category 0 |  |  | ± 0.3 dB |  |  | Same as 9.2.1.1 |  |  |
| 9.7.2.1 FDD and Half duplex FDD CQI reporting definition under fading conditions for UE category 0 |  |  | ± 0.6 dB |  |  | Same as 9.3.1.1.1 |  |  |
| 9.7.2.2 TDD CQI reporting definition under fading conditions for UE category 0 |  |  | ± 0.6 dB |  |  | Same as 9.3.1.1.1 |  |  |
| 9.7_1.1.1 FDD CQI reporting under AWGN conditions - PUCCH 1-0 for UE category 1bis |  |  | ± 0.3 dB |  |  | Same as 9.2.1.1 |  |  |
| 9.7_1.1.2 TDD CQI reporting under AWGN conditions - PUCCH 1-0 for UE category 1bis |  |  | ± 0.3 dB |  |  | Same as 9.2.1.1 |  |  |
| 9.7_1.1.3 FDD CQI reporting under fading conditions - PUSCH 3-0 for UE category 1bis |  |  | ± 0.6 dB |  |  | Same as 9.3.1.1.1 |  |  |
| 9.7_1.1.4 TDD CQI reporting under fading conditions - PUSCH 3-0 for UE category 1bis |  |  | ± 0.6 dB |  |  | Same as 9.3.1.1.1 |  |  |
| 9.8.1.1 FDD and Half duplex FDD CQI reporting definition under AWGN conditions for UE category M1 |  |  | ± 0.3 dB |  |  | Same as 9.2.1.1 |  |  |
| 9.8.1.2 TDD CQI reporting definition under AWGN conditions for UE category M1 |  |  | ± 0.3 dB |  |  | Same as 9.2.1.1 |  |  |
| 9.8.2.1 FDD and Half duplex FDD CQI reporting definition under fading conditions for UE category M1 |  |  | ± 0.6 dB |  |  | Same as 9.3.1.1.1 |  |  |
| 9.8.2.2 TDD CQI reporting definition under fading conditions for UE category M1 |  |  | ± 0.6 dB |  |  | Same as 9.3.1.1.1 |  |  |
| 9.9.1.1.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 with Rank 1 1x4 |  |  | ± 0.3 dB |  |  | Same as 9.2.1.1 |  |  |
| 9.9.1.1.2 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 with Rank 1 1x4 |  |  | ± 0.3 dB |  |  | Same as 9.2.1.1 |  |  |
| 9.9.1.2.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-1 with rank 2 4x4 |  |  | ± 0.3 dB |  |  | Same as 9.2.1.1 |  |  |
| 9.9.1.2.2 TDD CQI Reporting under AWGN conditions – PUCCH 1-1 with rank 2 8x4 |  |  | ± 0.3 dB |  |  | Same as 9.2.1.1 |  |  |
| 9.9.1.3.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-1 with rank 4 4x4 |  |  | ± 0.3 dB |  |  | Same as 9.2.1.1 |  |  |
| 9.9.1.3.2 TDD CQI Reporting under AWGN conditions – PUCCH 1-1 with rank 4 4x4 |  |  | ± 0.3 dB |  |  | Same as 9.2.1.1 |  |  |
| 9.9.1.4.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-1 with rank 3 4x4 |  |  | ± 0.3 dB |  |  | Same as 9.2.1.1 |  |  |
| 9.9.1.4.2 TDD CQI Reporting under AWGN conditions – PUCCH 1-1 with rank 3 8x4 |  |  | ± 0.3 dB |  |  | Same as 9.2.1.1 |  |  |
| 9.9.2.1.1 FDD CQI Reporting under fading conditions - PUCCH 1-0 - Enhanced Performance Requirement Type A 1x4 |  |  | Ês1/Noc ± 0.6 dB Ês2/Noc ± 0.3 dB |  |  | Overall system uncertainty for fading conditions on Cell 1 comprises two quantities: 1. Signal-to-noise ratio uncertainty ±0.3 dB 2. Fading profile power uncertainty ±0.5 dB for single Tx Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect AWGN absolute power uncertainty ±3.0 dB not expected to have any significant effect |  |  |
| 9.9.2.1.2 TDD CQI Reporting under fading conditions - PUCCH 1-0 - Enhanced Performance Requirement Type A 1x4 |  |  | Ês1/Noc ± 0.6 dB Ês2/Noc ± 0.3 dB |  |  | Same as 9.9.2.1.1 |  |  |
| 9.9.2.2.1 FDD CQI Reporting under fading conditions - PUCCH 1-1 - Enhanced Performance Requirement Type A 2x4 |  |  | Ês1/Noc ± 0.8 dB Ês2/Noc ± 0.3 dB |  |  | Overall system uncertainty for fading conditions on Cell 1 comprises two quantities: 1. Signal-to-noise ratio uncertainty ±0.3 dB 2. Fading profile power uncertainty ±0.7 dB for two Tx antennas. Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect AWGN absolute power uncertainty ±3.0 dB not expected to have any significant effect |  |  |
| 9.9.2.2.2 TDD CQI Reporting under fading conditions - PUCCH 1-1 - Enhanced Performance Requirement Type A 2x4 |  |  | Ês1/Noc ± 0.8 dB Ês2/Noc ± 0.3 dB |  |  | Same as 9.9.2.2.1 |  |  |
| 9.9.3.1.1 TDD PMI Reporting – PUSCH 3-1 (Single PMI) 8x4 |  |  | ± 0.6 dB |  |  | Same as 9.4.1.1.1 |  |  |
| 9.9.4.1.1 FDD RI Reporting– PUCCH 1-1 4x4 |  |  | ± 0.8 dB |  |  | Same as 9.5.1.1 |  |  |
| 9.9.4.1.2 TDD RI Reporting– PUSCH 3-1 4x4 |  |  | ± 0.8 dB |  |  | Same as 9.5.1.1 |  |  |
| 9.9.4.2.1 FDD RI Reporting- PUCCH 1-1 for eDL-MIMO 1x4 |  |  | ± 0.8 dB |  |  | Same as 9.5.1.1 |  |  |
| 9.9.4.2.2 TDD RI Reporting- PUCCH 1-1 for eDL-MIMO 1x4 |  |  | ± 0.8 dB |  |  | Same as 9.5.1.1 |  |  |
| 9.10.1.1 FDD CSI-RS Resource Indicator Reporting – PUSCH 3-1 |  |  | ± 0.6 dB |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty ±0.3 dB 2. Fading profile power uncertainty ±0.5 dB Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect AWGN absolute power uncertainty ±3.0 dB not expected to have any significant effect |  |  |
| 9.10.1.2 FDD CSI-RS Resource Indicator Reporting – PUSCH 3-1 |  |  | ± 0.6 dB |  |  | Same as 9.10.1.1 |  |  |
| 9.12.1.1 FDD CQI Reporting under fading conditions (with cell-specific reference symbols configured for UE supporting short TTI) |  |  | ± 0.6 dB |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty ±0.3 dB 2. Fading profile power uncertainty ±0.5 dB Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect AWGN absolute power uncertainty ±3.0 dB not expected to have any significant effect |  |  |
| 9.12.1.1 TDD CQI Reporting under fading conditions (with cell-specific reference symbols configured for UE supporting short TTI) |  |  | ± 0.6 dB |  |  | Same as 9.12.1.1 |  |  |
| 9.12.2.1 FDD CQI Reporting under fading conditions (with CSI reference symbols configured for UE supporting short TTI) |  |  | ± 0.6 dB |  |  | Same as 9.12.1.1 |  |  |
| 9.12.2.2 FDD CQI Reporting under fading conditions (with CSI reference symbols configured for UE supporting short TTI) |  |  | ± 0.6 dB |  |  | Same as 9.12.1.1 |  |  |
| In addition, the following Test System uncertainties and related constraints apply: |  |  |  |  |  |  |  |  |
| AWGN Bandwidth |  |  |  |  |  | ≥ 1.08MHz, 2.7MHz, 4.5MHz, 9MHz, 13.5MHz, 18MHz; NRB x 180kHz according to BWConfig |  |  |
| AWGN absolute power uncertainty, averaged over BWConfig Note 4 |  |  |  |  |  | ±3 dB |  |  |
| AWGN flatness and signal flatness, max deviation for any Resource Block, relative to average over BWConfig |  |  |  |  |  | ±2 dB |  |  |
| AWGN peak to average ratio |  |  |  |  |  | ≥10 dB @0.001% |  |  |
| Signal-to noise ratio uncertainty, averaged over downlink transmission Bandwidth |  |  |  |  |  | ±0.3 dB (includes uncertainty in precoding applied by the test system, where applicable) |  |  |
| Signal-to noise ratio variation for any resource block, relative to average over downlink transmission Bandwidth |  |  |  |  |  | ±0.5 dB |  |  |
| Noc2 absolute power uncertainty, averaged over BWConfig Note 5 |  |  |  |  |  | ±3 dB |  |  |
| Noc1 / Noc2 ratio uncertainty, averaged over BWConfig Note 5 |  |  |  |  |  | ±0.3 dB |  |  |
| Noc3 / Noc2 ratio uncertainty, averaged over BWConfig Note 5 |  |  |  |  |  | ±0.3 dB |  |  |
| Es / Noc2 ratio (SNR) uncertainty, averaged over downlink transmission Bandwidth Note 5 |  |  |  |  |  | ±0.3 dB (includes uncertainty in precoding applied by the test system, where applicable) |  |  |
| Fading profile power uncertainty |  |  |  |  |  | Test-specific |  |  |
| Fading profile delay uncertainty, relative to frame timing |  |  |  |  |  | ±5 ns (excludes absolute errors related to baseband timing) |  |  |
| Downlink channel matrix uncertainties: |  |  |  |  |  |  |  |  |
| Tx phase error, as shown in Figure F.1.5-1 |  |  |  |  |  | is defined for each Tx antenna up to NTx ≤10° is constant for the duration of the test |  |  |
| Rx phase error, as shown in Figure F.1.5-1 |  |  |  |  |  | is defined for each Rx antenna up to NRx There is no constraint on the value of except that it is constant for the duration of the test |  |  |
| Note 1: Only the overall stimulus error is considered. The effect of errors in the throughput measurements due to finite test duration is not considered. Note 2: The AWGN parameters apply to all test cases except 9.3.3.1.1 and 9.3.3.1.2. The fading parameters apply to test cases using fading Note 3: Downlink channel matrix uncertainties apply to eDL-MIMO CSI test cases Note 4: Applies for test cases which specify Noc, a single value that remains constant with time. Note 5: Applies for test cases which specify Noc1, Noc2 and Noc3, that are symbol or subframe specific. |  |  |  |  |  |  |  |  |

Figure F.1.5-1: Location of Tx Phase error and Rx Phase error for eDL-MIMO (Informative)

# F.2	Interpretation of measurement results (normative)

The Test Requirements in the present document have been calculated by relaxing the Minimum Requirements of the core specification using the Test Tolerances (TT) defined in F.3. The measurement results returned by the Test System are compared – without any modification – against the Test Requirements as defined by either the “Never fail a good DUT” principle for Test Tolerance equal measurement uncertainty (TT = MU) or “Shared Risk” principle for Test Tolerance equal to 0 (TT = 0).

The “Never fail a good DUT” and the “Shared Risk” principles are defined in ETR 273-1-2 clause 6.5.

The actual measurement uncertainty of the Test System for the measurement of each parameter shall be included in the test report.

The recorded value for the Test System uncertainty shall be, for each measurement, equal to or lower than the appropriate figure in clause F.1 of the present document.

If the Test System for a test is known to have a measurement uncertainty greater than that specified in clause F.1, it is still permitted to use this apparatus provided that an adjustment is made value as follows:

Any additional uncertainty in the Test System over and above that specified in clause F.1 shall be used to tighten the Test Requirement, making the test harder to pass. For some tests, for example receiver tests, this may require modification of stimulus signals. This procedure will ensure that a Test System not compliant with clause F.1does not increase the chance of passing a device under test where that device would otherwise have failed the test if a Test System compliant with clause F.1 had been used.

# F.3	Test Tolerance and Derivation of Test Requirements (informative)

The Test Requirements in the present document have been calculated by relaxing the Minimum Requirements of the core specification using the Test Tolerances defined in this clause. When the Test Tolerance is zero, the Test Requirement will be the same as the Minimum Requirement. When the Test Tolerance is non-zero, the Test Requirements will differ from the Minimum Requirements, and the formula used for the relaxation is given in this clause.

The Test Tolerances are derived from Test System uncertainties, regulatory requirements and criticality to system performance. As a result, the Test Tolerances may sometimes be set to zero.

The test tolerances should not be modified for any reason e.g. to take account of commonly known test system errors (such as mismatch, cable loss, etc.).

The downlink Test Tolerances apply at each receiver antenna connector.

## F.3.1	Measurement of test environments

The UE test environments are set to the values defined in TS 36.508 subclause 4.1, without any relaxation. The applied Test Tolerance is therefore zero.

## F.3.2	Measurement of transmitter

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Test |  | Minimum Requirement in TS 36.101 |  |  | Test Tolerance (TT) | Test Requirement in TS 36.521-1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6.2.2 UE Maximum Output Power |  | f ≤ 3.0GHz Power class 1: [FFS] Power class 2: [FFS] Power class 3: 23dBm ±2 dB Power class 4: [FFS] 3.0GHz < f ≤ 4.2GHz Power class 3: 23dBm +2/-3 dB 4.2GHz < f ≤ 6GHz Power class 3: 23dBm ±2 dB |  |  | 0.7 dB 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.3 dB | Formula: Upper limit + TT, Lower limit - TT Power class 1: [FFS] Power class 2: [FFS] Power class 3: 23dBm ±2.7 dB Power class 4: [FFS] Power class 3: 23dBm +3.0/-4.0 dB Power class 3: 23dBm ±3.3 dB |  |
| 6.2.2_1 UE Maximum Output Power for HPUE |  | f ≤ 3.0GHz Power class 1: 31dBm +2/-3dB Power class 2: 26dBm ±2dB 3.0GHz < f ≤ 4.2GHz Power class 2: 26 dBm +2/-3 dB |  |  | 0.7dB 1.0 dB | Formula: Upper limit + TT, Lower limit - TT Power class 1: 31dBm +2.7/-3.7dB Power class 2: 26dBm ±2.7dB Power class 2: 26 dBm +3.0/-4.0 dB |  |
| 6.2.2A.1 UE Maximum Output Power for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.2.2 |  |  | Same as 6.2.2 | Same as 6.2.2 |  |
| 6.2.2A.2 UE Maximum Output Power for CA (inter-band DL CA and UL CA) |  | Same as 6.2.2 |  |  | Same as 6.2.2 | Same as 6.2.2 |  |
| 6.2.2B UE Maximum Output Power for UL-MIMO |  | f ≤ 3.0GHz Power class 1: [FFS] Power class 2: [FFS] Power class 3: 23dBm +2/-3 dB Power class 4: [FFS] 3.0GHz < f ≤ 4.2GHz Power class 3: 23dBm +2/-4 dB |  |  | Same as 6.2.2 | Formula: Upper limit +TT, Lower limit - TT Power class 1: [FFS] Power class 2: [FFS] Power class 3: 23dBm +2.7/-3.7 dB Power class 4: [FFS] Power class 3: 23dBm +3.0/-5.0 dB Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.2.2B_1 HPUE Maximum Output Power for UL-MIMO |  | f ≤ 3.0GHz Power class 1: 31dBm +2/-3 dB Power class 2: 26dBm +2/-3 dB 3.0GHz < f ≤ 4.2GHz Power class 2: 26dBm +2/-4 dB |  |  | Same as 6.2.2 | Formula: Upper limit + TT, Lower limit - TT Power class 1: 31dBm +2.7/-3.7dB Power class 2: 26dBm +2.7/-3.7dB Power class 2: 26 dBm +3.0/-5.0 dB |  |
| 6.2.2E UE Maximum Output Power UE category 0 |  | Same as 6.2.2 |  |  | Same as 6.2.2 | Same as 6.2.2 |  |
| 6.2.2EA UE Maximum Output Power UE category M1 |  | f ≤ 3.0GHz Power class 3: 23dBm ±2 dB Power class 5: 20dBm ±2 dB Power class 6: 14dBm ±2.5 dB 3.0GHz < f ≤ 4.2GHz Power class 3: 23dBm ±2 dB Power class 5: 20dBm ±2 dB Power class 6: 14dBm ±2.5 dB |  |  | 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB | Formula: Upper limit + TT, Lower limit - TT Power class 3: 23dBm ±2.7 dB Power class 5: 20dBm ±2.7 dB Power class 6: 14dBm ±3.2 dB Power class 3: 23dBm ±3.0 dB Power class 5: 20dBm ±3.0 dB Power class 6: 14dBm ±3.5 dB |  |
| 6.2.2EB UE Maximum Output Power UE category 1bis |  | Same as 6.2.2 |  |  | Same as 6.2.2 | Same as 6.2.2 |  |
| 6.2.2EC UE Maximum Output Power for UE category M2 |  | Same as 6.2.2EA |  |  | Same as 6.2.2EA | Same as 6.2.2EA |  |
| 6.2.2F UE Maximum Output Power for UE category NB1 and NB2 |  | f ≤ 3.0GHz Power class 1: [FFS] Power class 2: [FFS] Power class 3: 23dBm ±2 dB Power class 4: [FFS] Power class 5: 20dBm ±2 dB 3.0GHz < f ≤ 4.2GHz Power class 1: [FFS] Power class 2: [FFS] Power class 3: 23dBm ±2 dB Power class 4: [FFS] Power class 5: 20dBm ±2 dB |  |  | 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB | Formula: Upper limit + TT, Lower limit - TT Power class 1: [FFS] Power class 2: [FFS] Power class 3: 23dBm ±2.7 dB Power class 4: [FFS] Power class 5: 20dBm ±2.7 dB Power class 1: [FFS] Power class 2: [FFS] Power class 3: 23dBm ±3.0 dB Power class 4: [FFS] Power class 5: 20dBm ±3.0 dB |  |
| 6.2.2FA UE Maximum Output Power for category NB1 and NB2/Power Class 6 |  | f ≤ 3.0GHz Power class 6: 14dBm ±2.5dB 3.0GHz < f ≤ 4.2GHz Power class 6: 14dBm ±2.5dB |  |  | 0.7dB 1.0 dB | Formula: Upper limit + TT, Lower limit - TT Power class 6: 14dBm ±3.2dB Power class 6: 14dBm ±3.5dB |  |
| 6.2.2G.1 UE maximum output power for V2X Commincation / Non-concurrent with E-UTRA uplink transmissions |  | 4.2GHz < f ≤ 6GHz Power class 3: 23dBm ±2 dB |  |  | 1.3 dB | Formula: Upper limit + TT, Lower limit - TT Power class 3: 23dBm ±3.3 dB |  |
| 6.2.2G.2 UE maximum output power for V2X Commincation / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission |  | 4.2GHz < f ≤ 6GHz Power class 3: 23dBm +2/-3 dB |  |  | 1.3 dB | Formula: Upper limit + TT, Lower limit - TT Power class 3: 23dBm +3.3/-4.3 dB |  |
| 6.2.2G.3 UE maximum output power for V2X Commincation / Intra-band contiguous multi-carrier operation |  | 4.2GHz < f ≤ 6GHz Power class 3: 23dBm ±2 dB |  |  | 1.3 dB | Formula: Upper limit + TT, Lower limit - TT Power class 3: 23dBm ±3.3 dB |  |
| 6.2.2G.4.1 UE maximum output power for V2X Commincation / Power class 2 / Non-concurrent with E-UTRA uplink transmissions |  | 4.2GHz < f ≤ 6GHz Power class 2: 26dBm ±2 dB |  |  | 1.3 dB | Formula: Upper limit + TT, Lower limit - TT Power class 2: 26dBm ±3.3 dB |  |
| 6.2.2_s UE Maximum Output Power for subslot/slot TTI |  | Same as 6.2.2 |  |  | Same as 6.2.2 | Same as 6.2.2 |  |
| 6.2.3 Maximum Power Reduction |  | Power class 3 f ≤ 3.0GHz QPSK: MPR ≤ 1dB 16QAM: MPR ≤ 1dB 16QAM: MPR ≤ 2dB 3.0GHz < f ≤ 4.2GHz QPSK: MPR ≤ 1dB 16QAM: MPR ≤ 1dB 16QAM: MPR ≤ 2dB |  |  | 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB | Formula: Upper limit + TT, Lower limit – MPR – TT Power class 3: QPSK: 23dBm +2.7 / - 3.7dB 16QAM: 23dBm +2.7 / - 3.7dB 16QAM: 23dBm +2.7 / - 4.7dB QPSK: 23dBm +3.0 / - 5.0dB 16QAM: 23dBm +3.0 / - 5.0dB 16QAM: 23dBm +3.0 / - 6.0dB |  |
| 6.2.3_1 Maximum Power Reduction for HPUE |  | Power class 1 and 2 f ≤ 3.0GHz QPSK: MPR ≤ 1dB 16QAM: MPR ≤ 1dB 16QAM: MPR ≤ 2dB Power class 2: 3.0GHz < f ≤ 4.2GHz QPSK: MPR ≤ 1 dB 16QAM: MPR ≤ 1 dB 16QAM: MPR ≤ 2 dB |  |  | 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB | Formula: Upper limit + TT, Lower limit – MPR – TT Power class 1: QPSK: 31dBm +2.7 / - 4.7dB 16QAM: 31dBm +2.7 / - 4.7dB 16QAM: 31dBm +2.7 / - 5.7dB Power class 2: QPSK: 26dBm +2.7 / - 3.7dB 16QAM: 26dBm +2.7 / - 3.7dB 16QAM: 26dBm +2.7 / - 4.7dB QPSK: 26dBm +3.0 / -5.0 dB 16QAM: 26dBm +3.0 / -5.0 dB 16QAM: 26dBm +3.0 / -6.0 dB |  |
| 6.2.3_2 UE Maximum Output Power for Multi-Cluster PUSCH |  | Same as 6.2.2 |  |  | Same as 6.2.2 | Same as 6.2.2 |  |
| 6.2.3_3 Maximum Power Reduction (MPR) for UL 64QAM |  | Power class 3 f ≤ 3.0GHz 64QAM: MPR ≤ 2dB 64QAM: MPR ≤ 3dB 3.0GHz < f ≤ 4.2GHz 64QAM: MPR ≤ 2dB 64QAM: MPR ≤ 3dB |  |  | 0.7 dB 0.7 dB 1.0 dB 1.0 dB | Formula: Upper limit + TT, Lower limit – MPR – TT Power class 3: 64QAM: 23dBm +2.7 / - 4.7dB 64QAM: 23dBm +2.7 / - 6.2dB 64QAM: 23dBm +3.0 / - 5.0dB 64QAM: 23dBm +3.0 / - 6.5dB |  |
| 6.2.3_4 Maximum Power Reduction (MPR) for Multi-Cluster PUSCH with UL 64QAM |  | Same as 6.2.3_3 |  |  | Same as 6.2.3_3 | Same as 6.2.3_3 |  |
| 6.2.3_5 Maximum Power Reduction (MPR) for UL 256QAM |  | Power class 3 f ≤ 3.0GHz 256QAM: MPR ≤ 5dB 3.0GHz < f ≤ 4.2GHz 256QAM: MPR ≤ 5dB |  |  | 0.7 dB 1.0 dB | Formula: Upper limit + TT, Lower limit – MPR – TT Power class 3: 256QAM: 23dBm +2.7 / - 7.7dB 256QAM: 23dBm +3.0 / - 8.0dB |  |
| 6.2.3_6 Maximum Power Reduction (MPR) for Multi-Cluster PUSCH with UL 256QAM |  | Power class 3 f ≤ 3.0GHz 256 QAM: MPR = 5.5dB 256 QAM: MPR = 6 dB 256 QAM: MPR = 7 dB 256 QAM: MPR = 7.5dB 3.0GHz < f ≤ 4.2GHz 256 QAM: MPR = 5.5dB 256 QAM: MPR = 6 dB 256 QAM: MPR = 7 dB 256 QAM: MPR = 7.5dB |  |  | 0.7 dB 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB | Formula: Upper limit: +T(PCMAX_H_C) + TT, Lower limit: -T(PCMAX_L_C) – MPR – TT PCMAX_H_C = PPowerClass PCMAX_L_C = PPowerClass-MPR Power class 3: 256QAM: 23dBm +2.7 / - 11.2dB 256QAM: 23dBm +2.7 / - 11.7dB 256QAM: 23dBm +2.7 / - 12.7dB 256QAM: 23dBm +2.7 / - 13.2dB 256QAM: 23dBm +2.7 / - 11.5dB 256QAM: 23dBm +2.7 / - 12.0dB 256QAM: 23dBm +2.7 / - 13.0dB 256QAM: 23dBm +2.7 / - 13.5dB |  |
| 6.2.3A.1 Maximum Power Reduction (MPR) for CA (intra-band contiguous DL CA and UL CA) |  | Power class 3 f ≤ 3.0GHz MPR = 0dB MPR = 1dB MPR = 2dB MPR = 3dB MPR = 8.5dB MPR = 10dB MPR = 10.5dB 3.0GHz < f ≤ 4.2GHz MPR = 0dB MPR = 1dB MPR = 2dB MPR = 3dB MPR = 8.5dB MPR = 10.0dB MPR = 10.0dB |  |  | 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -max(TL,c, T(PCMAX_L )) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass-MPR Power class 3: TL,c=2dB 23dBm +2.7 / - 2.7dB 23dBm +2.7 / - 3.7dB 23dBm +2.7 / - 4.7dB 23dBm +2.7 / - 6.2dB 23dBm +2.7 / - 14.2dB 23dBm +2.7 / - 15.7dB 23dBm +2.7 / - 17.2dB TL,c=3dB 23dBm +3.0 / - 4.0dB 23dBm +3.0 / - 5.0dB 23dBm +3.0 / - 6.0dB 23dBm +3.0 / - 7.5dB 23dBm +3.0 / - 14.5dB 23dBm +3.0 / - 16.0dB 23dBm +3.0 / - 17.5dB |  |
| 6.2.3A.1_1 Maximum Power Reduction (MPR) for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM |  | Power class 3 f ≤ 3.0GHz MPR = 2dB MPR = 3dB 3.0GHz < f ≤ 4.2GHz MPR = 2dB MPR = 3dB |  |  | 0.7 dB 0.7 dB 1.0 dB 1.0 dB | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass-MPR Power class 3: 23dBm +2.7 / - 4.7dB 23dBm +2.7 / - 6.2dB 23dBm +3.0 / - 5.0dB 23dBm +3.0 / - 6.5dB |  |
| 6.2.3A.1_2 Maximum Power Reduction (MPR) for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM |  | Power class 3 f ≤ 3.0GHz MPR = 5dB 3.0GHz < f ≤ 4.2GHz MPR = 5dB |  |  | 0.7 dB 1.0 dB | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass-MPR Power class 3: 23dBm +2.7 / - 9.7dB 23dBm +3.0 / - 10.0dB |  |
| 6.2.3A.2 Maximum Power Reduction (MPR) for CA (inter-band DL CA and UL CA) |  | Power class 3 f ≤ 3.0GHz MPR = 0dB MPR = 1dB MPR = 2dB 4.2GHz < f ≤ 6GHz MPR = 0dB MPR = 1dB MPR = 2dB |  |  | 0.7 dB 0.7 dB 0.7 dB 1.5 dB 1.5 dB 1.5 dB | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = MIN(linear sum of PPowerClass – MPR c -·TC,c, 23 dBm) Power class 3: 23dBm +2.7 / - 3.7 dB 23dBm +2.7 / - 3.7 dB 23dBm +2.7 / - 3.7 dB 23dBm +3.5 / - 4.5 dB 23dBm +3.5 / - 5.5 dB 23dBm +3.5 / - 6.5 dB |  |
| 6.2.3A.2_1 Maximum Power Reduction (MPR) for CA (inter-band DL CA and UL CA) for UL 64QAM |  | Power class 3 f ≤ 3.0GHz MPR = 2dB MPR = 3dB |  |  | 0.7 dB 0.7 dB | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – TT PCMAX_H = PPowerClass PCMAX_L = MIN(linear sum of PPowerClass – MPR c -·TC,c, 23 dBm) Power class 3: 23dBm +2.7 / - 5.7 dB 23dBm +2.7 / - 6.7 dB |  |
| 6.2.3A.2_2 Maximum Power Reduction (MPR) for CA (inter-band DL CA and UL CA) for UL 256QAM |  | Power class 3 f ≤ 3.0GHz MPR = 5dB 3.0GHz < f ≤ 4.2GHz MPR = 5dB |  |  | 0.7 dB 1.0 dB | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – TT PCMAX_H = PPowerClass PCMAX_L = MIN(linear sum of PPowerClass – MPR c -·TC,c, 23 dBm) Power class 3: 23dBm +2.7 / - 7.7 dB 23dBm +2.7 / - 9.4 dB 23dBm +3.0 / - 8.0 dB 23dBm +3.0 / - 9.7 dB |  |
| 6.2.3A.3 Maximum Power Reduction (MPR) for CA (intra-band non-contiguous DL CA and UL CA) |  | Power class 3 f ≤ 3.0GHz MPR = 7 dB MPR = 12 dB MPR = 18 dB |  |  | 0.7 dB 0.7 dB 0.7 dB | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass – MPR -·TC dBm) Power class 3: 23dBm +2.7 / - 12.7 dB 23dBm +2.7 / - 18.7 dB 23dBm +2.7 / - 25.7 dB |  |
| 6.2.3A.3_1 Maximum Power Reduction (MPR) for CA (intra-band non-contiguous DL CA and UL CA) for UL 64QAM |  | Power class 3 f ≤ 3.0GHz MPR = 7 dB MPR = 12 dB MPR = 18 dB |  |  | 0.7 dB 0.7 dB 0.7 dB | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass – MPR -·TC dBm) Power class 3: 23dBm +2.7 / - 12.7 dB 23dBm +2.7 / - 18.7 dB 23dBm +2.7 / - 25.7 dB |  |
| 6.2.3A.3_2 Maximum Power Reduction (MPR) for CA (intra-band non-contiguous DL CA and UL CA) for UL 256QAM |  | Power class 3 f ≤ 3.0GHz MPR = 7 dB MPR = 12 dB MPR = 18 dB |  |  | 0.7 dB 0.7 dB 0.7 dB | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass – MPR -·DTC dBm) Power class 3: 23dBm +2.7 / - 12.7 dB 23dBm +2.7 / - 18.7 dB 23dBm +2.7 / - 25.7 dB |  |
| 6.2.3A.4 Maximum Power Reduction (MPR) for CA (3UL CA) |  | Power class 3 f ≤ 3.0GHz for intra-band MPR = 0dB MPR = 1dB MPR = 2dB MPR = 3dB MPR = 3.5dB 3.0GHz < f ≤ 4.2GHz for intra-band MPR = 0dB MPR = 1dB MPR = 2dB MPR = 3dB MPR = 3.5dB f ≤ 3.0GHz for inter-band MPR = 0dB MPR = 1dB MPR = 1dB MPR = 2dB MPR = 2dB 3.0GHz < f ≤ 4.2GHz for intra-band MPR = 0dB MPR = 1dB MPR = 1dB MPR = 2dB MPR = 2dB |  |  | 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 1 dB 1 dB 1 dB 1 dB 1 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 1 dB 1 dB 1 dB 1 dB 1 dB | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass-MPR Power class 3: 23dBm +2.7 / - 2.7dB 23dBm +2.7 / - 3.7dB 23dBm +2.7 / - 4.7dB 23dBm +2.7 / - 5.2dB 23dBm +2.7 / - 6.2dB 23dBm +3 / -3 dB 23dBm +3 / -4 dB 23dBm +3 / -5 dB 23dBm +3 / -5.5 dB 23dBm +3 / -6.5 dB Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = MIN(linear sum of PPowerClass – MPR c -·TC,c, 23 dBm) Power class 3: 23dBm +2.7 / - 3.7 dB 23dBm +2.7 / - 3.7 dB 23dBm +2.7 / - 5.7 dB 23dBm +2.7 / - 3.7 dB 23dBm +2.7 / - 5.7 dB 23dBm +3 / - 4 dB 23dBm +3 / - 4 dB 23dBm +3 / - 6 dB 23dBm +3 / - 4 dB 23dBm +3 / - 6 dB |  |
| 6.2.3B Maximum Power Reduction (MPR) for UL-MIMO |  | Same as 6.2.3 |  | Same as 6.2.3 |  | Same as 6.2.3 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.2.3E Maximum Power Reduction for UE category 0 |  | Same as 6.2.3E |  | Same as 6.2.3E |  | Same as 6.2.3E |  |
| 6.2.3EA Maximum Power Reduction for UE category M1 |  | Power class 3 f ≤ 3.0GHz MPR ≤ 1dB MPR ≤ 2dB 3.0GHz < f ≤ 4.2GHz MPR ≤ 1dB MPR ≤ 2dB Power class 5 f ≤ 3.0GHz MPR ≤ 1dB MPR ≤ 2dB 3.0GHz < f ≤ 4.2GHz MPR ≤ 1dB MPR ≤ 2dB Power class 6 f ≤ 3.0GHz MPR = 0dB MPR = 1dB MPR = 2dB 3.0GHz < f ≤ 4.2GHz MPR = 0dB MPR = 1dB MPR = 2dB |  | 0.7 dB 0.7 dB 1.0 dB 1.0 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB |  | Formula: Upper limit + TT, Lower limit – MPR – TT 23dBm +2.7 / - 3.7dB 23dBm +2.7 / - 4.7dB 23dBm +3.0 / - 4.0dB 23dBm +3.0 / - 5.0dB 20dBm +2.7 / - 3.7dB 20dBm +2.7 / - 4.7dB 20dBm +3.0 / - 4.0dB 20dBm +3.0 / - 5.0dB 14dBm +3.2 / - 3.2dB 14dBm +3.2 / - 4.2dB 14dBm +3.2 / - 6.2dB 14dBm +3.5 / - 3.5dB 14dBm +3.5 / - 4.5dB 14dBm +3.5 / - 6.5dB |  |
| 6.2.3EB Maximum Power Reduction for UE category 1bis |  | Same as 6.2.3 |  | Same as 6.2.3 |  | Same as 6.2.3 |  |
| 6.2.3F Maximum Power Reduction (MPR) for UE category NB1 and NB2 |  | Power class 3: f ≤ 3.0GHz BPSK: MPR = 0dB QPSK: MPR = 0dB QPSK: MPR ≤ 0.5dB QPSK: MPR ≤ 1dB QPSK: MPR ≤ 2dB 3.0GHz < f ≤ 4.2GHz BPSK: MPR = 0dB QPSK: MPR = 0dB QPSK: MPR ≤ 0.5dB QPSK: MPR ≤ 1dB QPSK: MPR ≤ 2dB Power class 5: f ≤ 3.0GHz BPSK: MPR = 0dB QPSK: MPR = 0dB QPSK: MPR ≤ 0.5dB QPSK: MPR ≤ 1dB QPSK: MPR ≤ 2dB 3.0GHz < f ≤ 4.2GHz BPSK: MPR = 0dB QPSK: MPR = 0dB QPSK: MPR ≤ 0.5dB QPSK: MPR ≤ 1dB QPSK: MPR ≤ 2dB |  | 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB |  | Formula: Upper limit + TT, Lower limit – MPR – TT Power class 3: BPSK: 23dBm +2.7 / - 2.7dB QPSK: 23dBm +2.7 / - 2.7dB QPSK: 23dBm +2.7 / - 3.2dB QPSK: 23dBm +2.7 / - 3.7dB QPSK: 23dBm +2.7 / - 4.7dB BPSK: 23dBm +3.0 / - 3.0dB QPSK: 23dBm +3.0 / - 3.0dB QPSK: 23dBm +3.0 / - 3.5dB QPSK: 23dBm +3.0 / - 4.0dB QPSK: 23dBm +3.0 / - 5.0dB Power class 5: BPSK: 20dBm +2.7 / - 3.2dB QPSK: 20dBm +2.7 / - 3.2dB QPSK: 20dBm +2.7 / - 4.7dB QPSK: 20dBm +2.7 / - 5.2dB QPSK: 20dBm +2.7 / - 6.7dB BPSK: 20dBm +3.0 / - 3.5dB QPSK: 20dBm +3.0 / - 3.5dB QPSK: 20dBm +3.0 / - 5.0dB QPSK: 20dBm +3.0 / - 5.5dB QPSK: 20dBm +3.0 / - 7.0dB |  |
| 6.2.3G.1.1 Maximum Power Reduction (MPR) for V2X Commincation / Non-concurrent with E-UTRA uplink transmissions |  | Power class 3: 4.2GHz < f ≤ 6GHz QPSK: MPR ≤ 1.5dB 16QAM: MPR ≤ 2dB |  | 1.3 dB 1.3 dB |  | Formula: Upper limit + TT, Lower limit – MPR – TT Power class 3: QPSK: 23dBm +3.3 / - 4.8dB 16QAM: 23dBm +3.3 / - 5.3dB |  |
| 6.2.3G.1.2 Maximum Power Reduction (MPR) for V2X Communication / Power class 3 / Non-contiguous allocation of PSCCH and PSSCH / Non-concurrent with E-UTRA uplink transmissions |  | Power class 3: 5 RBs/10MHz: MPR ≤4.5dB 25 RBs/10MHz: MPR ≤3.0dB 40 RBs/10MHz: MPR ≤2.0dB 10 RBs/20MHz: MPR ≤4.5dB 50 RBs/20MHz: MPR ≤3.0dB 80 RBs/20MHz: MPR ≤2.0dB |  | 1.3 dB 1.3 dB 1.3 dB 1.3 dB 1.3 dB 1.3 dB |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass-MPR Power class 3: 23dBm + 3.3 / - 9.8dB 23dBm + 3.3 / - 6.8dB 23dBm + 3.3 / - 5.3dB 23dBm + 3.3 / - 9.8dB 23dBm + 3.3 / - 6.8dB 23dBm + 3.3 / - 5.3dB |  |
| 6.2.3G.1.3 Maximum Power Reduction (MPR) for V2X Communication / Power class 3 / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission |  | Power class 3 MPR ≤ 1.5dB MPR ≤ 2dB |  | 1.3 dB 1.3 dB |  | Formula: Upper limit + THIGH (PCMAX_H(p, q)) + TT, Lower limit – TLOW (PCMAX_L(p, q)) – MPR – ΔTIB,c – TT Power class 3: 23dBm +3.3 / -7.8dB 23dBm +3.3 / -8.3dB |  |
| 6.2.3G.1.4 Maximum Power Reduction (MPR) for V2X Communication / Power class 3 / Intra-band contiguous multi-carrier operation |  | Power class 3 MPR ≤ 3dB |  | 1.3 dB |  | Formula: Upper limit + THIGH (PCMAX_H(p, q)) + TT, Lower limit – TLOW (PCMAX_L(p, q)) – MPR – ΔTIB,c – TT Power class 3: 23dBm +3.3 / -6.8dB |  |
| 6.2.3G.2.1 Maximum Power Reduction (MPR) for V2X Communication / Power class 2 / Contiguous allocation of PSCCH and PSSCH / Non-concurrent with E-UTRA uplink transmissions |  | Power class 2: QPSK : MPR ≤ 2dB 16QAM : MPR ≤ 2.5dB |  | 1.3 dB 1.3 dB |  | Formula: Upper limit + TT, Lower limit – MPR – TT Power class 2: QPSK : 26dBm +3.3 / -5.3dB 16QAM : 26dBm +3.3 / -5.8dB |  |
| 6.2.3_s Maximum Power Reduction (MPR) for subslot/slot TTI |  | Same as 6.2.3 |  | Same as 6.2.3 |  | Same as 6.2.3 |  |
| 6.2.3G.2.2 Maximum Power Reduction (MPR) for V2X Communication / Power class 2 / Non-contiguous allocation of PSCCH and PSSCH / Non-concurrent with E-UTRA uplink transmissions |  | Power class 2 4.2GHz < f ≤ 6GHz For 10MHz BW: NRB_alloc 10: MPR = 4.5dB NRB_alloc 12: MPR = 4dB NRB_alloc 42: MPR = 2.5dB For 20MHz BW: NRB_alloc 10: MPR = 9dB NRB_alloc 22: MPR = 5.5dB NRB_alloc 82: MPR = 3dB |  | 1.3 dB 1.3 dB 1.3 dB 1.3 dB 1.3 dB 1.3 dB |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass-MPR Power class 2: For 10MHz BW: NRB_alloc 10: 26dBm +3.3 / -7.8dB NRB_alloc 12: 26dBm +3.3 / -7.3dB NRB_alloc 42: 26dBm +3.3 / -5.8dB For 20MHz BW: NRB_alloc 10: 26dBm +3.3 / -15.3dB NRB_alloc 22: 26dBm +3.3 / -9.3dB NRB_alloc 82: 26dBm +3.3 / -6.3dB |  |
| 6.2.4 UE Maximum Output Power with additional requirements |  | For the UE maximum output power modified by MPR and A-MPR, the power limits specified in TS 36.101 [2] clause 6.2.5 apply. For transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. Power class 3: QPSK: MPR ≤ 1dB 16QAM: Depending on the number RB allocated: 16QAM: MPR ≤ 1dB 16QAM: MPR ≤ 2dB For network signalled value NS_03 , NS_04 (5MHz only), NS_05, to NS_06: A-MPR ≤ 1dB For network signalled value NS-04; Depending on the RB_start and RB allocation (10MHz, 15MHz and 20MHz): For 10MHz Region A with RB_start=0 – 12: A-MPR ≤ 3dB. Region B with RB_start=13 – 36 : A-MPR ≤ 2dB. Region C with RB_start=37 – 49 : A-MPR ≤ 3dB. For 15MHz Region A with RB_start=0 – 18: A-MPR ≤ 3dB. Region B with RB_start=19 – 55 : A-MPR ≤ 2dB. Region C with RB_start=56 – 74 : A-MPR ≤ 3dB. For 20MHz Region A with RB_start=0 – 24: A-MPR ≤ 3dB. Region B with RB_start=25 – 74 : A-MPR ≤ 2dB. Region C with RB_start=75 – 99 : A-MPR ≤ 3dB. For network signalled value NS-11; Depending on the RB_start and RB allocation (15MHz and 20MHz): For 15MHz Fc < 2012.5 Region A with RB_start=0 – 4: A-MPR ≤ 15dB. Region B with RB_start=5 – 21: A-MPR ≤ 7dB (LCRB=7-50) or ≤ 10dB (LCRB=0-6 & ≥50). Region C with RB_start=22 – 56: A-MPR ≤ 0dB (LCRB=≤25) or ≤ 6dB (LCRB>25). Region D with RB_start=57 – 74: A-MPR ≤ 15dB. For 15MHz Fc = 2012.5 Region A with RB_start=0 – 12: A-MPR ≤ 10dB. Region B with RB_start=13 – 39: A-MPR ≤ 6dB (LCRB=≥ 30) or ≤ 0dB (LCRB=< 30). Region C with RB_start=40 – 65: A-MPR ≤ 2dB. Region D with RB_start=66 – 74: A-MPR ≤ 6.5dB. For 20MHz Region A with RB_start=0 – 12: A-MPR ≤ 15dB. Region B with RB_start=13 – 29: A-MPR ≤ 7dB (LCRB=10-60) or ≤ 10dB (LCRB=1-9 & > 60). Region C with RB_start=30 – 68: A-MPR ≤ 0dB (LCRB=1-24) or ≤ 7dB (LCRB= ≥ 25). Region D with RB_start=69 – 99: A-MPR ≤ 15dB. For network signalled value NS-20; Depending on the RB_start and RB allocation (5MHZ, 10MHz, 15MHz and 20MHz): For 5MHz Region A with RB_start=≤24: A-MPR ≤ 17dB. Region B with RB_start=0 – 3: A-MPR ≤ 1dB (LCRB=15-19) or ≤ 4dB (LCRB= ≥ 20). Region C with RB_start=4 – 6: A-MPR ≤ 2dB. Region D with RB_start=≤24: A-MPR ≤ 0dB. For 10MHz Fc = 2005 Region A with RB_start=0 – 25: A-MPR ≤ 16dB. Region B with RB_start=26 – 34: A-MPR ≤ 2dB (LCRB=8-15) or ≤ 5dB (LCRB= > 15). Region C with RB_start=35 – 49: A-MPR ≤ 6dB. For 10MHz Fc = 2015 Region A with RB_start=0 – 5: A-MPR ≤ 4dB. Region B with RB_start=6 – 10: A-MPR ≤ 2dB. For 15MHz Region A with RB_start=0 – 14: A-MPR ≤ 11dB (LCRB=1-9 & 40-5) or ≤ 6dB (LCRB= 10-39). Region B with RB_start=15 – 24: A-MPR ≤ 1dB (LCRB=24-29) or ≤ 7dB (LCRB=≥30). Region C with RB_start=25 – 39: A-MPR ≤ 5dB. Region D with RB_start=61 – 74: A-MPR ≤ 6dB. For 20MHz Region A with RB_start=0 – 21: A-MPR ≤ 17dB. Region B with RB_start=22 – 31: A-MPR ≤ 12dB (LCRB=1-9 & 31-75) or ≤ 6dB (LCRB=10-30). Region C with RB_start=32 – 38: A-MPR ≤ 9dB. Region D with RB_start=39 – 49: A-MPR ≤ 7dB. Region E with RB_start=50 – 69: A-MPR ≤ 5dB. Region F with RB_start=70 – 99: A-MPR ≤ 16dB. For network signalled value NS-27; Depending on the RB_start and RB allocation (15MHz and 20MHz): For 15MHz With LCRB ≤15, LRB_start=0 – 6 or RB_end=68-74: A-MPR ≤ 4dB. With LCRB ≥ 60, LRB_start≥0: A-MPR ≤ 2dB. For 20MHz With LCRB ≤20, LRB_start=0 – 12 or RB_end=87-99: A-MPR ≤ 4dB. With LCRB ≤3, LRB_start=13 – 15 or RB_end=84-86: A-MPR ≤ 1dB. With LCRB ≥ 60, LRB_start≥0: A-MPR ≤ 3dB. For network signalled value NS_56; Depending on the RB_start and RB allocation (5MHz and 10MHz): For 5MHz Region A with RB_start≤8 and LCRB ≤8: A-MPR ≤ 8dB. Region B with RB_start≤8 and LCRB >8: A-MPR ≤ 2dB. For 10MHz Region A with RB_start≤5 and LCRB ≤5: A-MPR ≤ 7dB. Region B with RB_start≤18 and LCRB ≤12: A-MPR ≤ 5dB. Region C with RB_start≤18 and LCRB >12: A-MPR ≤ 7dB. Region D with RB_start≥35 and LCRB ≤7: A-MPR ≤ 4dB. Region E with RB_start≥35 and LCRB >7: A-MPR ≤ 2dB. Region F with RB_start≥40 and LCRB ≤7: A-MPR ≤ 5dB. Region G with RB_start≥40 and LCRB >7: A-MPR ≤ 3dB. |  | f ≤ 3.0GHz 0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB |  | Formula: Upper limit + TT, A: Lower limit – TT, B: (UE Maximum Output Power from 6.2.2) - T(PCMAX) – MPR – TT, C: (UE Maximum Output Power from 6.2.2) - T(PCMAX) – A-MPR – TT, D: (UE Maximum Output Power from 6.2.2) - T(PCMAX) – A-MPR – MPR – TT E: (UE Maximum Output Power from 6.2.2) – TL – A-MPR – TT, F: (UE Maximum Output Power from 6.2.2) – TL – A-MPR – MPR – TT Power class 3: Test Requirement Configuration ID versus Formula Above Network signalled value NS_03: [A]: 2, 5, 10, 15, 20, 25 [B]: 1, 3, 7 [C]: 9, 14, 19, 24 [D]: 4, 6, 8, 11, 12, 13, 16, 17, 18, 21, 22, 23, 26, 27 Network signalled value NS_04 (5, 10, 15, 20MHz): [A] 3 [B] 10, 11, 19, 20, 28, 29 [C] 2, 6, 7, 13, 14, 15, 16, 23, 24, 25, 32 [D] 1, 4, 5, 8, 9, 12, 17, 18, 21, 22, 26, 27, 30, 31 Network signalled value NS_05: [A] 1, 3, 4, 8, 9, 14, 15, [B] 2, 5, 10, 11, 16, 17 [C] None [D] 6, 7, 12, 13, 18, 19 Network signalled value NS_06: [A]: 2, 5, 8, 11, 14, 17 [B]: 1, 3, 4, 6, 7, 9, 10, 12, 13, 15, 16, 18 [C]: None [D]: None Network signalled value NS_07: [A]:3, 8, 12 [B]: 7, 9 [C]: 1, 2, 5, 13, 15 [D]: 4, 6, 10, 11, 14, 16 Network signalled value NS_08: [A]:1, 2, 4, 5, 12 [B]: 3, 6, 11, 13 [C]: None [D]: 7, 8, 9, 10, 14, 15, 16, 17 Network signalled value NS_11: [A]: 5c, 6b, 10c, 11c, 25 [B]: 8b, 12c, 13c, 14c, 17a, 17b, 21a, 21b, 28 [C]: 5a, 5b, 6a, 6c, 10a, 10b, 11, 11b, 15a, 15b, 16a, 16b, 24 [D]: 1a, 1b, 2a, 2b, 3a, 3b, 4a, 4b ,, 7a,7b,7c, 8a, 8c, 9a, 9b,9c, 12a, 12b, 13a, 13b, 14a, 14b, 18a, 18b, 19a, 19b, 20a, 20b, 22a, 22b, 23a, 23b, 26, 27, 29, 30 Network signalled value NS_12: [A]: 3, 8, 13 [B]: None [C]: 1, 4, 6, 11 [D]: 2, 5, 7, 9, 10, 12, 14, 15 Network signalled value NS_13: [A]: None [B]: 3, 4 [C]: 1 [D]: 2, 5 Network signalled value NS_14: [A]: None [B]: 2, 4, 7, 9 [C]: 1, 6 [D]: 3, 5, 8, 10 Network signalled value NS_15: [A]: 1, 7, 14 [B]: 3, 6, 10, 15, 19, 21, 22, 25 [C]: 12, 13 [D]: 2, 4, 5, 8, 9, 11, 16, 17, 18, 20, 23, 24, 26 Network signalled value NS_20: [A]: 1b, 1c, 1d, 6b, 7b [B]: 2d, 3d, 4d, 5d, 9b [C]: 1a, 6a, 7a, 11, 18, 19 [D]: 2a, 2b, 2c, 3a, 3b, 3c, 4a, 4b, 4c, 5a, 5b, 5c, 8a, 8b, 9a, 10a, 11b, 12, 13, 14, 15, 16, 17, 20, 21, 22, 23, 24, 25, 26 Network signalled value NS_27: TBD Network signalled value NS_56 A: None B: None C: 1, 4, 5, 8, 9, 10, 11, D: 3, 6, 7, 14 E: 13 F: 2, 12 Network signalled value NS_62 [B]: 1, 2, 3, 4, 5, 6, 7, 8, 9 |  |
| 6.2.4_1 Additional Maximum Power Reduction (A-MPR) for HPUE |  | For the UE maximum output power modified by MPR and A-MPR, the power limits specified in TS 36.101 [2] clause 6.2.5 apply. For transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. Power class 1: QPSK: MPR ≤ 1dB 16QAM: Depending on the number RB allocated: 16QAM: MPR ≤ 1dB 16QAM: MPR ≤ 2dB Power class 1: For network signalled value NS_06: A-MPR 0dB |  | 0.7dB |  | Formula: Upper limit + TT, (UE Maximum Output Power from 6.2.2) - T(PCMAX) – A-MPR – MPR – TT |  |
| 6.2.4_2 Additional Maximum Power Reduction (A-MPR) for UL 64QAM |  | Same as 6.2.4 |  | Same as 6.2.4 |  | Same as 6.2.4 |  |
| 6.2.4_3 Additional Maximum Power Reduction (A-MPR) with PUSCH frequency hopping |  | Same as 6.2.4 |  | Same as 6.2.4 |  | Same as 6.2.4 |  |
| 6.2.4_4 Additional Maximum Power Reduction (A-MPR) for UL 256QAM |  | For the UE maximum output power modified by MPR and A-MPR, the power limits specified in TS 36.101 [2] clause 6.2.5 apply. For transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. Power class 3: 256QAM: MPR ≤ 5dB A-MPR values as given by tables in section 6.2.4.3. |  | f ≤ 3.0GHz 0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB |  | Formula: Upper limit + TT, A: (UE Maximum Output Power from 6.2.2) - T(PCMAX) - MPR - TT, B: (UE Maximum Output Power from 6.2.2) - T(PCMAX) - A-MPR - MPR - TT Power class 3: Test Requirement Configuration ID versus Formula Above Network signalled value NS_03 [A]: 1, 3 [B]: 2, 4, 5, 6, 7, 8, 9, 10, 11 Network signalled value NS_04 [A]: 1, (2, 3, 4, 5) if not low range [B]: (2, 3, 4, 5) for low range Network signalled value NS_05 [A]: 1, 5, 6 [B]: 2, 3, 4, 7 Network signalled value NS_06 [A]: 1, 2, 3 [B]: None Network signalled value NS_07 [A]: 2 [B]: 1, 3, 4 Network signalled value NS_08 [A]: 1 [B]: 2, 3 Network signalled value NS_09 [A]: 1 [B]: 2, 3 Network signalled value NS_11 [A]: 3b, 5c, 6c, 8a with high range RB allocation [B]: 1a, 1b, 2a, 2b, 3a, 3c, 4a, 4b, 4c, 5a, 5b, 6a, 6b, 7a, 7b, 8a with low range RB allocation, 8b, 9a, 9b, 10a, 10b, 11, 12, 13 Network signalled value NS_12 [A]: None [B]: 1, 2, 3, 4 Network signalled value NS_13 [A]: None [B]: 1 Network signalled value NS_14 [A]: None [B]: 1, 2 Network signalled value NS_15 [A]: None [B]: 1, 2, 3, 4, 5 Network signalled value NS_16 [A]: 1, 2 for FC = 810.0 MHz, 3 for FC = 814.5 MHz [B]: 2 for FC < 810.0 MHz, 3 for FC < 814.5 MHz, 4 Network signalled value NS_17 [A]: 1, 2 [B]: None Network signalled value NS_18 [A]: None [B]: 1, 2, 3, 4 Network signalled value NS_19 [A]: 1, (2, 3, 4) with high range RB allocation [B]: (2, 3, 4) with low range RB allocation Network signalled value NS_20 [A]: 1b with high range RB allocation, 1c, 1d, 2c, 2d, 3a with high range RB allocation, 3b, 5a with high range RB allocation [B]: 1a, 1b with low range RB allocation, 2a, 2b, 3a with low range RB allocation, 4a, 4b, 5a with low range RB allocation, 6a, 7a, 8a, 9a, 10a, 11a Network signalled value NS_21 [A]: None [B]: 1 Network signalled value NS_22 [A]: 1, 2 [B]: 3, 4, 5, 6, 7, 8 Network signalled value NS_23 [A]: 1, 2 [B]: None Network signalled value NS_24 [A]: 1, 3, 5 [B]: 2, 4, 6 Network signalled value NS_25 [A]: 6 [B]: 1, 2, 3, 4, 5 Network signalled value NS_27 [A]: 1, 2 [B]: 3, 4 Network signalled value NS_35 [A]: 1, 2, 3, 4 [B]: None Network signalled value NS_36 [A]: 2, 4, 6 [B]: 1, 3, 5 Network signalled value NS_56 [A]: None [B]: 1, 2, 3, 4 Network signalled value NS_62 [A]: 1, 2, 3, 4, 5, 6 |  |
| 6.2.4A.1 Additional Maximum Power Reduction (A-MPR) for CA (intra-band contiguous DL CA and UL CA) |  | A-MPR values as detailed in core requirement |  | Same as 6.2.4 |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L )– A-MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass-A-MPR |  |
| 6.2.4A.1_1 Additional Maximum Power Reduction (A-MPR) for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM |  | Same as 6.2.4A.1 |  | Same as 6.2.4A.1 |  | Same as 6.2.4A.1 |  |
| 6.2.4A.1_2 Additional Maximum Power Reduction (A-MPR) for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM |  | Same as 6.2.4A.1 |  | Same as 6.2.4A.1 |  | Same as 6.2.4A.1 |  |
| 6.2.4A.2 Additional Maximum Power Reduction (A-MPR) for CA (inter-band DL CA and UL CA) |  | Same as 6.2.4 |  | Same as 6.2.4 |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – AMPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass – MPR – AMPR -TC |  |
| 6.2.4A.2_1 Additional Maximum Power Reduction (A-MPR) for CA (inter-band DL CA and UL CA) for UL 64QAM |  | Same as 6.2.4 |  | Same as 6.2.4 |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – AMPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass – MPR – AMPR -TC |  |
| 6.2.4A.2_2 Additional Maximum Power Reduction (A-MPR) for CA (inter-band DL CA and UL CA) for UL 256QAM |  | Same as 6.2.4 |  | Same as 6.2.4 |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – AMPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass – MPR – AMPR -TC |  |
| 6.2.4A.4 Additional Maximum Power Reduction (A-MPR) for CA (3UL CA) |  | A-MPR values as detailed in core requirement |  | Same as 6.2.4 |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L )– A-MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass-A-MPR |  |
| 6.2.4B Additional Maximum Power Reduction (A-MPR) for UL-MIMO |  | Same as 6.2.4 |  | Same as 6.2.4 |  | Same as 6.2.4 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.2.4E Additional Maximum Power Reduction (A-MPR) for UE category 0 |  | Same as 6.2.4 |  | Same as 6.2.4 |  | Same as 6.2.4 |  |
| 6.2.4EA Additional Maximum Power Reduction (A-MPR) for UE category M1 |  | A-MPR values as detailed in core requirement |  | Same as 6.2.4 |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L )– A-MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass-A-MPR |  |
| 6.2.4EB Additional Maximum Power Reduction (A-MPR) for UE category 1bis |  | Same as 6.2.4 |  | Same as 6.2.4 |  | Same as 6.2.4 |  |
| 6.2.4EC Additional Maximum Power Reduction (A-MPR) for UE category M2 |  | Same as 6.2.4EA |  | Same as 6.2.4EA |  | Same as 6.2.4EA |  |
| 6.2.4G.1 Additional Maximum Power Reduction (A-MPR) for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | A-MPR values as detailed in core requirement |  | Same as 6.2.4 |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – AMPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass – MPR – AMPR -TC |  |
| 6.2.4G.2 Additional Maximum Power Reduction (A-MPR) for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | A-MPR values as detailed in core requirement |  | 1.3dB |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – AMPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass – MPR – AMPR -TC |  |
| 6.2.4_s Additional Maximum Power Reduction (A-MPR) for subslot/slot TTI |  | Same as 6.2.4 |  | Same as 6.2.4 |  | Same as 6.2.4 |  |
| 6.2.5 Configured UE transmitted Output Power |  | f ≤ 3.0GHz 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 3.0GHz < f ≤ 4.2GHz 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 |  | 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB |  | Formula: Upper limit + TT, Lower limit – TT 13 ≤ PCMAX < 18 ± 5.7 8 ≤ PCMAX < 13 ± 6.7 -40 ≤ PCMAX < 8 ± 7.7 13 ≤ PCMAX < 18 ± 6.0 8 ≤ PCMAX < 13 ± 7.0 -40 ≤ PCMAX < 8 ± 8.0 |  |
| 6.2.5_s Configured UE Transmitted Output Power for subslot/slot TTI |  | Same as 6.2.5 |  | Same as 6.2.5 |  | Same as 6.2.5 |  |
| 6.2.5_1 Configured UE transmitted Output Power for HPUE |  | f ≤ 3.0GHz 23 ≤ PCMAX < 33 ± 2.0 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 3.0GHz < f ≤ 4.2GHz 23 ≤ PCMAX < 33 ± 2.0 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 |  | 0.7 dB 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB |  | Formula: Upper limit + TT, Lower limit – TT 23 ≤ PCMAX < 33 ± 2.7 13 ≤ PCMAX < 18 ± 5.7 8 ≤ PCMAX < 13 ± 6.7 -40 ≤ PCMAX < 8 ± 7.7 23 ≤ PCMAX < 33 +3/-4 13 ≤ PCMAX < 18 ± 6 8 ≤ PCMAX < 13 ± 7 -40 ≤ PCMAX < 8 ± 8 |  |
| 6.2.5A.1 Configured UE transmitted Output Power for CA (intra-band contiguous DL CA and UL CA) |  | f ≤ 3.0GHz 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 3.0GHz < f ≤ 4.2GHz 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 |  | 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB |  | Formula: Upper limit + TT, Lower limit – TT 13 ≤ PCMAX < 18 ± 5.7 8 ≤ PCMAX < 13 ± 6.7 -40 ≤ PCMAX < 8 ± 7.7 13 ≤ PCMAX < 18 ± 6.0 8 ≤ PCMAX < 13 ± 7.0 -40 ≤ PCMAX < 8 ± 8.0 |  |
| 6.2.5A.1_3 Configured UE transmitted Output Power for CA and HPUE (intra-band contiguous DL CA and UL CA) for HPUE |  | f ≤ 3.0GHz 23 ≤ PCMAX < 33 ± 2.0 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 3.0GHz < f ≤ 4.2GHz 23 ≤ PCMAX < 33 ± 2.0 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 |  | 0.7 dB 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB |  | Formula: Upper limit + TT, Lower limit – TT 23 ≤ PCMAX < 33 ± 2.7 13 ≤ PCMAX < 18 ± 5.7 8 ≤ PCMAX < 13 ± 6.7 -40 ≤ PCMAX < 8 ± 7.7 23 ≤ PCMAX < 33 ± 3 13 ≤ PCMAX < 18 ± 6 8 ≤ PCMAX < 13 ± 7 -40 ≤ PCMAX < 8 ± 8 |  |
| 6.2.5A.3 Configured UE transmitted Output Power for CA (inter-band DL CA and UL CA) |  | f ≤ 3.0GHz 16 ≤ PCMAX < 20 ± 5.0 11≤ PCMAX < 16 ± 6.0 -40 ≤ PCMAX < 11 ± 7.0 |  | 0.7 dB 0.7 dB 0.7 dB |  | Formula: Upper limit + TT, Lower limit – TT 16 ≤ PCMAX < 20 ± 5.7 11 ≤ PCMAX < 16 ± 6.7 -40 ≤ PCMAX < 11 ± 7.7 |  |
| 6.2.5A.4 Configured UE transmitted Output Power for CA (intra-band non-contiguous DL CA and UL CA) |  | f ≤ 3.0GHz 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 |  | 0.7 dB 0.7 dB 0.7 dB |  | Formula: Upper limit + TT, Lower limit – TT 13 ≤ PCMAX < 18 ± 5.7 8 ≤ PCMAX < 13 ± 6.7 -40 ≤ PCMAX < 8 ± 7.7 |  |
| 6.2.5B Configured UE transmitted output power for UL-MIMO |  | f ≤ 3.0GHz [16] ≤ PCMAX < [20] ± [5.0] [11] ≤ PCMAX < [16] ± [6.0] [-40] ≤ PCMAX < [11] ± [7.0] 3.0GHz < f ≤ 4.2GHz [16] ≤ PCMAX < [20] ± [5.0] [11] ≤ PCMAX < [16] ± [6.0] [-40] ≤ PCMAX < [11] ± [7.0] |  | Same as 6.2.5 |  | Formula: Upper limit + TT, Lower limit – TT [16] ≤ PCMAX < [20] ± [5.7] [11] ≤ PCMAX < [16] ± [6.7] [-40] ≤ PCMAX < [11] ± [7.7] [16] ≤ PCMAX < [20] ± [6.0] [11] ≤ PCMAX < [16] ± [7.0] [-40] ≤ PCMAX < [11] ± [8.0] Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.2.5E Configurated Transmitted Power for UE category 0 |  | Same as 6.2.5 |  | Same as 6.2.5 |  | Same as 6.2.5 |  |
| 6.2.5EA Configured UE transmitted Power for UE category M1 |  | Same as 6.2.5, with addition of power class 6: f ≤ 3.0GHz 5 ≤ PCMAX < 8 ± 6.0 -40 ≤ PCMAX < 5 ± 7.0 3.0GHz < f ≤ 4.2GHz 5 ≤ PCMAX < 8 ± 6.0 -40 ≤ PCMAX < 5 ± 7.0 |  | Same as 6.2.5, with addition of power class 6: 0.7 dB 0.7 dB 1.0 dB 1.0 dB |  | Same as 6.2.5, with addition of power class 6: Formula: Upper limit + TT, Lower limit – TT f ≤ 3.0GHz 5 ≤ PCMAX < 8 ± 6.7 -40 ≤ PCMAX < 5 ± 7.7 3.0GHz < f ≤ 4.2GHz 5 ≤ PCMAX < 8 ± 7.0 -40 ≤ PCMAX < 5 ± 8.0 |  |
| 6.2.5EB Configured UE transmitted Power for UE category 1bis |  | Same as 6.2.5 |  | Same as 6.2.5 |  | Same as 6.2.5 |  |
| 6.2.5F Configured UE transmitted Output Power for UE category NB1 and NB2 |  | Tolerances as described in core requirements |  | 0.7 dB |  | Formula: Upper limit + TT, Lower limit – TT |  |
| 6.2.5G.1 Configured UE transmitted Output Power for V2X Communication / Non-concurrent with E-UTRA uplink transmission |  | 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 |  | 1.3 dB 1.3 dB 1.5 dB |  | Formula: Upper limit + TT, Lower limit - TT 13 ≤ PCMAX < 18 ± 6.3 8 ≤ PCMAX < 13 ± 7.3 -40 ≤ PCMAX < 8 ± 8.5 |  |
| 6.2.5G.2 Configured UE transmitted Output Power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission |  | 16 ≤ PCMAX < 20 ± 5.0 11 ≤ PCMAX < 16 ± 6.0 -40 ≤ PCMAX < 11 ± 7.0 |  | 1.3 dB 1.3 dB 1.5 dB |  | Formula: Upper limit + TT, Lower limit - TT 16 ≤ PCMAX < 20 ± 6.3 11 ≤ PCMAX < 16 ± 7.3 -40 ≤ PCMAX < 11 ± 8.5 |  |
| 6.2.5G.3 Configured UE transmitted Output Power for V2X Communication / Intra-band contiguous multi-carrier operation |  | 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 |  | 1.3 dB 1.3 dB 1.5 dB |  | Formula: Upper limit + TT, Lower limit - TT 13 ≤ PCMAX < 18 ± 6.3 8 ≤ PCMAX < 13 ± 7.3 -40 ≤ PCMAX < 8 ± 8.5 |  |
| 6.3.2 Minimum Output Power |  | f ≤ 3.0GHz -40 dBm 3.0GHz < f ≤ 4.2GHz -40 dBm 4.2GHz < f ≤ 6GHz -40 dBm |  | 1.0 dB 1.3 dB 1.5 dB |  | Formula: Minimum Requirement + TT UE min. output power =–39 dBm UE min. output power =–38.7 dBm UE min. output power =–38.5 dBm |  |
| 6.3.2A.1 Minimum Output Power for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.3.2 |  | Same as 6.3.2 |  | Same as 6.3.2 |  |
| 6.3.2A.4 Minimum Output Power for CA (3UL CA) |  | Same as 6.3.2 |  | Same as 6.3.2 |  | Same as 6.3.2 |  |
| 6.3.2B Minimum Output Power for UL-MIMO |  | Same as 6.3.2 |  | Same as 6.3.2 |  | Same as 6.3.2 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.3.2E Minimum Output Power for UE category 0 |  | Same as 6.3.2 |  | Same as 6.3.2 |  | Same as 6.3.2 |  |
| 6.3.2EA Minimum Output Power for UE category M1 |  | Same as 6.3.2 |  | Same as 6.3.2 |  | Same as 6.3.2 |  |
| 6.3.2EB Minimum Output Power for UE category 1bis |  | Same as 6.3.2 |  | Same as 6.3.2 |  | Same as 6.3.2 |  |
| 6.3.2F Minimum Output Power for UE category NB1 and NB2 |  | -40 dBm |  | 1.0 dB |  | Formula: Minimum Requirement + TT UE min. output power =–39 dBm |  |
| 6.3.2G.1 Minimum output power for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | 4.2GHz < f ≤ 6GHz -30 dBm |  | 1.5 dB |  | Formula: Minimum Requirement + TT UE min. output power =–28.5 dBm |  |
| 6.3.2G.2 Minimum output power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | f ≤ 3.0GHz -40 dBm 4.2GHz < f ≤ 6GHz -30 dBm |  | 1.0 dB 1.5 dB |  | Formula: Minimum Requirement + TT UE min. output power =–39 dBm UE min. output power =–28.5 dBm |  |
| 6.3.2G.3 Minimum output power for V2X Communication / Intra-band contiguous multi-carrier operation |  | 4.2GHz < f ≤ 6GHz -3 0 dBm |  | 1.5 dB |  | Formula: Minimum Requirement + TT UE min. output power =–28.5 dBm |  |
| 6.3.3 Transmission ON/OFF Power |  | f ≤ 3.0GHz -50 dBm 3.0GHz < f ≤ 4.2GHz -50 dBm 4.2GHz < f ≤ 6GHz -50 dBm |  | 1.5 dB 1.8 dB 2.0 dB |  | Formula: Minimum Requirement + TT UE OFF Power -48.5 dBm UE OFF Power -48.2 dBm UE OFF Power -48.0 dBm |  |
| 6.3.3A.1 UE Transmit OFF power for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.3.3 |  | Same as 6.3.3 |  | Same as 6.3.3 |  |
| 6.3.3A.2 Transmit OFF Power for CA (inter-band DL CA and UL CA) |  | Same as 6.3.3 |  | Same as 6.3.3 |  | Same as 6.3.3 |  |
| 6.3.3A.3 UE Transmit OFF power for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.3.3 |  | Same as 6.3.3 |  | Same as 6.3.3 |  |
| 6.3.3A.4 UE Transmit OFF power for CA (3UL CA) |  | Same as 6.3.3 |  | Same as 6.3.3 |  | Same as 6.3.3 |  |
| 6.3.3B UE Transmit OFF power for UL-MIMO |  | Same as 6.3.3 |  | Same as 6.3.3 |  | Same as 6.3.3 Uplink power measurement applies to each Tx antenna connector |  |
| 6.3.3D UE Transmit OFF power for ProSe |  | Same as 6.3.3 |  | Same as 6.3.3 |  | Same as 6.3.3 |  |
| 6.3.3E UE Transmit OFF Power for UE category 0 |  | Same as 6.3.3 |  | Same as 6.3.3 |  | Same as 6.3.3 |  |
| 6.3.3EA UE Transmit OFF power for UE category M1 |  | Same as 6.3.3 |  | Same as 6.3.3 |  | Same as 6.3.3 |  |
| 6.3.3EB UE Transmit OFF power for UE category 1bis |  | Same as 6.3.3 |  | Same as 6.3.3 |  | Same as 6.3.3 |  |
| 6.3.3F Transmit OFF power for category NB1 and NB2 |  | Same as 6.3.3 |  | Same as 6.3.3 |  | Same as 6.3.3 |  |
| 6.3.3G.1 UE Transmit OFF power for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | 4.2GHz < f ≤ 6GHz -50 dBm |  | 2.0 dB |  | Formula: Minimum Requirement + TT UE OFF Power -48 dBm |  |
| 6.3.3G.2 UE Transmit OFF power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | f ≤ 3.0GHz -50 dBm 3.0GHz < f ≤ 4.2GHz -50 dBm 4.2GHz < f ≤ 6GHz -50 dBm |  | 1.5 dB 1.8 dB 2.0 dB |  | Formula: Minimum Requirement + TT UE OFF Power -48.5 dBm UE OFF Power -48.2 dBm UE OFF Power -48.0 dBm |  |
| 6.3.3G.3 UE Transmit OFF power for V2X Communication / Intra-band contiguous multi-carrier operation |  | 4.2GHz < f ≤ 6GHz -50 dBm |  | 2.0 dB |  | Formula: Minimum Requirement + TT UE OFF Power -48 dBm |  |
| 6.3.4.1 General ON/OFF time mask |  | f ≤ 3.0GHz OFF Power -50 dBm ON Power BW dependent 3.0GHz < f ≤ 4.2GHz OFF Power -50 dBm ON Power BW dependent Transmission ON Power value depends on the test parameters. In the particular test case parameters the ON power measurement has minimum requirements of ±6.0 dB |  | 1.5 dB 1.5 dB 1.8 dB 1.8 dB |  | Formula: OFF Power Minimum Req’t + TT ON Power Upper limit + TT, Lower limit – TT UE OFF Power -48.5 dBm UE ON Power: Test value ± 7.5 dB UE OFF Power -48.2 dBm UE ON Power: Test value ± 7.8 dB |  |
| 6.3.4.1_s General ON/OFF time mask for subslot/slot TTI |  | Same as 6.3.4.1 |  | Same as 6.3.4.1 |  | Same as 6.3.4.1 |  |
| 6.3.4.2 PRACH and SRS time mask |  | f ≤ 3.0GHz OFF Power -50 dBm ON Power BW dependent 3.0GHz < f ≤ 4.2GHz OFF Power -50 dBm ON Power BW dependent Transmission ON Power value depends on the test parameters. In the particular test case parameters the ON power measurement has minimum requirements of ±6.0 dB |  | 1.5 dB 1.5 dB 1.8 dB 1.8 dB |  | Formula: OFF Power Minimum Req’t + TT ON Power Upper limit + TT, Lower limit – TT UE OFF Power -48.5 dBm UE ON Power: Test value ± 7.5 dB UE OFF Power -48.2 dBm UE ON Power: Test value ± 7.8 dB |  |
| 6.3.4A.1.1 General ON/OFF time mask for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.3.4.1 for each CC |  | Same as 6.3.4.1 |  | Same as 6.3.4.1 for each CC |  |
| 6.3.4A.1.2 General ON/OFF time mask for CA (inter-band DL CA and UL CA) |  | Same as 6.3.4.1 |  | Same as 6.3.4.1 |  | Same as 6.3.4.1 |  |
| 6.3.4A.1.3 General ON/OFF time mask for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.3.4.1 for each CC |  | Same as 6.3.4.1 |  | Same as 6.3.4.1 for each CC |  |
| 6.3.4A.1.4 General ON/OFF time mask for CA (3UL CA) |  | Same as 6.3.4.1 for each CC |  | Same as 6.3.4.1 |  | Same as 6.3.4.1 for each CC |  |
| 6.3.4A.1.5 General ON/OFF time mask for CA (34UL CA) |  | Same as 6.3.4.1 for each CC |  | Same as 6.3.4.1 |  | Same as 6.3.4.1 for each CC |  |
| 6.3.4B.1 General ON/OFF time mask for UL-MIMO |  | Same as 6.3.4.1 |  | Same as 6.3.4.1 |  | Same as 6.3.4.1 Uplink OFF power measurement applies to each Tx antenna connector Uplink ON power measurement applies to [FFS] |  |
| 6.3.4C.1 General ON/OFF time mask for Dual Connectivity |  | Same as 6.3.4.1 |  | Same as 6.3.4.1 |  | Same as 6.3.4.1 |  |
| 6.3.4E.1 General ON/OFF time mask for UE category 0 |  | Same as 6.3.4.1 |  | Same as 6.3.4.1 |  | Same as 6.3.4.1 |  |
| 6.3.4E.2 PRACH and SRS time mask for UE category 0 |  | Same as 6.3.4.2 |  | Same as 6.3.4.2 |  | Same as 6.3.4.2 |  |
| 6.3.4EA.1 General ON/OFF time mask for UE category M1 |  | Same as 6.3.4.1 |  | Same as 6.3.4.1 |  | Same as 6.3.4.1 |  |
| 6.3.4EA.2 PRACH and SRS ON/OFF time mask for UE category M1 |  | Same as 6.3.4.2 |  | Same as 6.3.4.2 |  | Same as 6.3.4.2 |  |
| 6.3.4EB.1 General ON/OFF time mask for UE category 1bis |  | Same as 6.3.4.1 |  | Same as 6.3.4.1 |  | Same as 6.3.4.1 |  |
| 6.3.4EB.2 PRACH and SRS ON/OFF time mask for UE category 1bis |  | Same as 6.3.4.2 |  | Same as 6.3.4.2 |  | Same as 6.3.4.2 |  |
| 6.3.4EC.2 PRACH and SRS ON/OFF time mask for UE category M2 |  | Same as 6.3.4.2 |  | Same as 6.3.4.2 |  | Same as 6.3.4.2 |  |
| 6.3.4F.1 General ON/OFF time mask for category NB1 and NB2 |  | Same as 6.3.4.1 |  | Same as 6.3.4.1 |  | Same as 6.3.4.1 |  |
| 6.3.4F.2 NPRACH time mask for category NB1 and NB2 |  | Same as 6.3.4.2 |  | Same as 6.3.4.2 |  | Same as 6.3.4.2 |  |
| 6.3.4G.1 General ON/OFF time mask for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | 4.2GHz < f ≤ 6GHz OFF Power -50 dBm ON Power BW dependent Transmission ON Power value depends on the test parameters. In the particular test case parameters the ON power measurement has minimum requirements of ±6.0 dB |  | 2.0dB 2.0dB |  | Formulae: OFF Power Minimum Req’t + TT ON Power Upper limit + TT, Lower limit – TT UE OFF Power -48 dBm UE ON Power: Test value ± 8.0 dB |  |
| 6.3.4G.2 General ON/OFF time mask for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | f ≤ 3.0GHz OFF Power -50 dBm ON Power BW dependent 3.0GHz < f ≤ 4.2GHz OFF Power -50 dBm ON Power BW dependent 4.2GHz < f ≤ 6.0GHz OFF Power -50 dBm ON Power BW dependent Transmission ON Power value depends on the test parameters. In the particular test case parameters the ON power measurement has minimum requirements of ±6.0 dB |  | 1.5 dB 1.5 dB 1.8 dB 1.8 dB 2.0 dB 2.0 dB |  | Formulae: OFF Power Minimum Req’t + TT ON Power Upper limit + TT, Lower limit – TT UE OFF Power -48.5 dBm UE ON Power: Test value ± 7.5 dB UE OFF Power -48.2 dBm UE ON Power: Test value ± 7.8 dB UE OFF Power -48.0 dBm UE ON Power: Test value ± 8.0 dB |  |
| 6.3.4G.3 General ON/OFF time mask for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 6.3.4G.1 for each CC |  | Same as 6.3.4G.1 |  | Same as 6.3.4G.1 for each CC |  |
| 6.3.4G.4 PSSS/SSSS time mask for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Same as 6.3.4G.1 |  | Same as 6.3.4G.1 |  | Same as 6.3.4G.1 |  |
| 6.3.5.1 Power Control Absolute power tolerance |  | f ≤ 3.0GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB 3.0GHz < f ≤ 4.2GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB 4.2GHz < f ≤ 6GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB |  | 1.0 dB 1.0 dB 1.4 dB 1.4 dB 2.0 dB 2.0 dB |  | Formula: Upper limit + TT, Lower limit – TT Normal conditions ± 10.0 dB Extreme conditions ± 13.0 dB Normal conditions ± 10.4 dB Extreme conditions ± 13.4 dB Normal conditions ± 11.0 dB Extreme conditions ± 14.0 dB |  |
| 6.3.5.2 Power Control Relative power tolerance |  | TS 36.101 [2] clause 6.3.5.1 All combinations of PUSCH and PUCCH transitions: ΔP < 2; ±2.5 dB 2 ≤ ΔP < 3; ±3.0 dB 3 ≤ ΔP < 4; ±3.5 dB 4 ≤ ΔP ≤ 10; ±4.0 dB 10 ≤ ΔP < 15; ±5.0 dB 15 ≤ ΔP; ±6.0 dB |  | 0.7 dB |  | Formula: Upper limit + TT, Lower limit – TT All combinations of PUSCH and PUCCH transitions: ΔP < 2; ±3.2 dB 2 ≤ ΔP < 3; ±3.7 dB 3 ≤ ΔP < 4; ±4.2 dB 4 ≤ ΔP < 10; ±4.7 dB 10 ≤ ΔP < 15; ±5.7 dB 15 ≤ ΔP; ±6.7 dB |  |
| 6.3.5.3 Aggregate power control tolerance |  | Aggregate power control tolerance within 21 ms: PUCCH = ±2.5 dB PUSCH = ±3.5 dB |  | 0.7 dB |  | Formula: Upper limit + TT, Lower limit - TT PUCCH = ±3.2 dB PUSCH = ±4.2 dB |  |
| 6.3.5_1.1 Power Control Absolute power tolerance for HPUE |  | f ≤ 3.0GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB 3.0GHz < f ≤ 4.2GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB |  | 1.0 dB 1.0 dB 1.4 dB 1.4 dB |  | Formula: Upper limit + TT, Lower limit – TT Normal conditions ± 10.0 dB Extreme conditions ± 13.0 dB Normal conditions ± 10.4 dB Extreme conditions ± 13.4 dB |  |
| 6.3.5_1.2 Power Control Relative power tolerance for HPUE |  | Same as 6.3.5.2 |  | Same as 6.3.5.2 |  | Same as 6.3.5.2 |  |
| 6.3.5_1.3 Aggregate power control tolerance for HPUE |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  |
| 6.3.5A.1.1 Power Control Absolute power tolerance for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.3.5.1 |  | Same as 6.3.5.1 |  | Same as 6.3.5.1 |  |
| 6.3.5A.1.2 Power Control Absolute power tolerance for CA (inter-band DL CA and UL CA) |  | Same as 6.3.5.1 |  | Same as 6.3.5.1 |  | Same as 6.3.5.1 |  |
| 6.3.5A.1.3 Power Control Absolute power tolerance for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.3.5.1 |  | Same as 6.3.5.1 |  | Same as 6.3.5.1 |  |
| 6.3.5A.1.4 Power Control Absolute power tolerance for CA(3UL CA) |  | Same as 6.3.5.1 |  | Same as 6.3.5.1 |  | Same as 6.3.5.1 |  |
| 6.3.5A.1.5 Power Control Absolute power tolerance for CA(SRS carrier based switching) |  | Same as 6.3.5.1 |  | Same as 6.3.5.1 |  | Same as 6.3.5.1 |  |
| 6.3.5A.1.6 Power Control Absolute power tolerance for CA(4UL CA) |  | Same as 6.3.5.1 |  | Same as 6.3.5.1 |  | Same as 6.3.5.1 |  |
| 6.3.5A.2.1 Power Control Relative power tolerance for CA (intra-band contiguous DL CA and UL CA) |  | TS 36.101 [2] clause 6.3.5A.2 refers to Table 6.3.5.2.1-1. All combinations of PUSCH and PUCCH transitions: SCC power increase PCC 0dB (ΔP ≤ 1dB): Normal 0 ±1.0dB Exceptions: 0 ±6.0dB SCC +9dB (4 ≤ ΔP ≤ 10): Normal +9 ±4.0dB Exceptions: +9 ±6.0dB SCC power decrease PCC 0dB (ΔP ≤ 1dB): Normal 0 ±1.0dB Exceptions: 0 ±6.0dB SCC -9dB (4 ≤ ΔP ≤ 10): Normal -9 ±4.0dB Exceptions: -9 ±6.0dB PCC and SCC power increase PCC +9dB (4 ≤ ΔP ≤ 10): Normal +9 ±4.0dB Exceptions: +9 ±6.0dB SCC +9dB (4 ≤ ΔP ≤ 10): Normal +9 ±4.0dB Exceptions: +9 ±6.0dB PCC and SCC power decrease PCC -9dB (4 ≤ ΔP ≤ 10): Normal -9 ±4.0dB Exceptions: -9 ±6.0dB SCC -9dB (4 ≤ ΔP ≤ 10): Normal -9 ±4.0dB Exceptions: -9 ±6.0dB Each CC can have exceptions |  | -0.7, +0.8dB -0.7, +0.8dB -0.8, +0.7dB -0.8, +0.8dB -0.8, +0.7dB -0.8, +0.7dB -0.8, +0.8dB -0.8, +0.8dB -0.7, +0.8dB -0.7, +0.8dB -0.7, +0.8dB -0.7, +0.8dB -0.8, +0.7dB -0.8, +0.7dB -0.8, +0.7dB -0.8, +0.7dB |  | Relative power measurement uncertainty comprises two quantities: 1. Uplink Relative level measurement uncertainty of Test system 2. IQ image Leakage onto weaker CC at -25dBc is allowed for the UE, so is added arithmetically. It can only increase the measured power of the weaker CC. UL relative meas’t uncert ±0.7dB Leakage +0.17dB max for 11dB CC power difference, based on -25dBc. All limits rounded to nearest 0.1dB. SCC power increase PCC: Normal -1.7, +1.8dB Exceptions: -6.7, +6.8dB SCC: Normal +4.2, +13.7dB Exceptions: +2.2, +15.8dB SCC power decrease PCC: Normal -1.8, +1.7dB Exceptions: -6.8, +6.7dB SCC: Normal -13.8, -4.2dB Exceptions: -15.8, -2.2dB PCC and SCC power increase PCC: Normal +4.3, +13.8dB Exceptions: +2.3, +15.8dB SCC: Normal +4.3, +13.8dB Exceptions: +2.3, +15.8dB PCC and SCC power decrease PCC: Normal -13.8, -4.3dB Exceptions: -15.8, -2.3dB SCC: Normal -13.8, -4.3dB Exceptions: -15.8, -2.3dB The analysis is recorded in 3GPP TR 36.904 [17] When setting SCC power for the next step, three factors are relevant: 1. Uplink Relative level measurement uncertainty 2. IQ image Leakage onto weaker CC at -25dBc is allowed for the UE 3) TPC step 1dB ±1dB is allowed for the UE These are added arithmetically, giving an extra margin of (1+0.7+0.17) = 1.87dB (2dB is allowed) to avoid the same power step exception being counted twice. |  |
| 6.3.5A.2.2 Power Control Relative power tolerance for CA (inter-band DL CA and UL CA) |  | Same as 6.3.5.2 on both PCC and SCC separately. |  | Same as 6.3.5.2 on both PCC and SCC separately. |  | Same as 6.3.5.2 on both PCC and SCC separately. |  |
| 6.3.5A.2.3 Power Control Relative power tolerance for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.3.5A.2.1 |  | Same as 6.3.5A.2.1 |  | Same as 6.3.5A.2.1 |  |
| 6.3.5A.2.4.1 Power Control Relative power tolerance for CA (intra-band contiguous 3DL CA and 3UL CA) |  | Same as 6.3.5A.2.1 |  | Same as 6.3.5A.2.1 |  | Same as 6.3.5A.2.1 |  |
| 6.3.5A.2.4.2 Power Control Relative power tolerance for CA (inter-band 3DL CA and 3UL CA) |  | Same as 6.3.5A.2.2 |  | Same as 6.3.5A.2.2 |  | Same as 6.3.5A.2.2 |  |
| 6.3.5A.2.4.3 Power Control Relative power tolerance for CA (intra-band non-contiguous 3DL CA and 3UL CA) |  | Same as 6.3.5A.2.3 |  | Same as 6.3.5A.2.3 |  | Same as 6.3.5A.2.3 |  |
| 6.3.5A.3.1 Aggregate power control tolerance for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  |
| 6.3.5A.3.2 Aggregate power control tolerance for CA (inter-band DL CA and UL CA) |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  |
| 6.3.5A.3.3 Aggregate power control tolerance for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  |
| 6.3.5A.3.4 Aggregate Power Control Tolerance for CA (3UL CA) |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  |
| 6.3.5A.3.5 Aggregate Power Control Tolerance for CA (4UL CA) |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  |
| 6.3.5B.1 Power Control Absolute Power Tolerance for UL- MIMO |  | Same as 6.3.5.1 |  | Same as 6.3.5.1 |  | Same as 6.3.5.1 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.3.5B.2 Power Control Relative power tolerance for UL-MIMO |  | Same as 6.3.5.2 |  | Same as 6.3.5.2 |  | Same as 6.3.5.2 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.3.5B.3 Aggregate power control tolerance for UL-MIMO |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.3.5C.2 Power Control Relative power tolerance for Dual Connectivity |  | Same as 6.3.5.2 on both PCC and SCC separately. |  | Same as 6.3.5.2 on both PCC and SCC separately. |  | Same as 6.3.5.2 on both PCC and SCC separately. |  |
| 6.3.5C.2_1 Power Control Relative power tolerance for asynchronous Dual Connectivity |  | Same as 6.3.5.2 on both PCC and SCC separately. |  | Same as 6.3.5.2 on both PCC and SCC separately. |  | Same as 6.3.5.2 on both PCC and SCC separately. |  |
| 6.3.5E.1 Power Control Absolute power tolerance for UE category 0 |  | Same as 6.3.5.1 |  | Same as 6.3.5.1 |  | Same as 6.3.5.1 |  |
| 6.3.5E.2 Power Control Relative power tolerance for UE category 0 |  | Same as 6.3.5.2 |  | Same as 6.3.5.2 |  | Same as 6.3.5.2 |  |
| 6.3.5E.3 Aggregate power control tolerance for UE category 0 |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  |
| 6.3.5EA.1 Power Control Absolute power tolerance for UE category M1 |  | f ≤ 3.0GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB 3.0GHz < f ≤ 4.2GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB |  | 1.0 dB 1.0 dB 1.4 dB 1.4 dB |  | Formula: Upper limit + TT, Lower limit – TT Normal conditions ± 10.0 dB Extreme conditions ± 13.0 dB Normal conditions ± 10.4 dB Extreme conditions ± 13.4 dB |  |
| 6.3.5EA.2 Power Control Relative power tolerance for UE category M1 |  | TS 36.101 [2] clause 6.3.5.1 All combinations of PUSCH and PUCCH transitions: ΔP < 2; ±2.5 dB 2 ≤ ΔP < 3; ±3.0 dB 3 ≤ ΔP < 4; ±3.5 dB 4 ≤ ΔP ≤ 10; ±4.0 dB 10 ≤ ΔP < 15; ±5.0 dB 15 ≤ ΔP; ±6.0 dB |  | 0.7 dB |  | Formula: Upper limit + TT, Lower limit – TT All combinations of PUSCH and PUCCH transitions: ΔP < 2; ±3.2 dB 2 ≤ ΔP < 3; ±3.7 dB 3 ≤ ΔP < 4; ±4.2 dB 4 ≤ ΔP < 10; ±4.7 dB 10 ≤ ΔP < 15; ±5.7 dB 15 ≤ ΔP; ±6.7 dB |  |
| 6.3.5EA.3 Aggregate power control tolerance for UE category M1 |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  |
| 6.3.5EA.3_1 Aggregate power control tolerance for UE category M1 (CE Mode B) |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  |
| 6.3.5EB.1 Power Control Absolute power tolerance for UE category 1bis |  | Same as 6.3.5.1 |  | Same as 6.3.5.1 |  | Same as 6.3.5.1 |  |
| 6.3.5EB.2 Power Control Relative power tolerance for UE category 1bis |  | Same as 6.3.5.2 |  | Same as 6.3.5.2 |  | Same as 6.3.5.2 |  |
| 6.3.5EB.3 Aggregate power control tolerance for UE category 1bis |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  |
| 6.3.5EC.3_1 Aggregate power control tolerance for UE category M2 (CE Mode B) |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  |
| 6.3.5F.1 Power Control Absolute power tolerance for category NB1 and NB2 |  | f ≤ 3.0GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB 3.0GHz < f ≤ 4.2GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB |  | 1.0 dB 1.0 dB 1.4 dB 1.4 dB |  | Formula: Upper limit + TT, Lower limit – TT Normal conditions ± 10.0 dB Extreme conditions ± 13.0 dB Normal conditions ± 10.4 dB Extreme conditions ± 13.4 dB |  |
| 6.3.5F.2 Power Control Relative power tolerance for UE category NB1 and NB2 |  | TS 36.101 [2] clause 6.3.5F.2 NPRACH transitions: ΔP = 0; ±1.5 dB ΔP = 2; ±2.0 dB ΔP = 4; ±3.5 dB ΔP = 6; ±4.0 dB |  | 0.7 dB |  | Formula: Upper limit + TT, Lower limit – TT NPRACH transitions: ΔP = 0; ±2.2 dB ΔP = 2; ±2.7 dB ΔP = 4; ±4.2 dB ΔP = 6; ±4.7 dB |  |
| 6.3.5F.3 Aggregate power control tolerance for category NB1 and NB2 |  | Aggregate power control tolerance within TBD ms: PUSCH = ±3.5 dB |  | 0.7 dB |  | Formula: Upper limit + TT, Lower limit - TT PUSCH = ±4.2 dB |  |
| 6.3.5G.1 Power Control Absolute power tolerance for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | 4.2GHz < f ≤ 6GHz Normal conditions ± 3.0 dB Extreme conditions ± 6.0 dB |  | 2.0 dB 2.0 dB |  | Formula: Upper limit + TT, Lower limit – TT Normal conditions ± 5.0 dB Extreme conditions ± 8.0 dB |  |
| 6.3.5G.2 Power Control Absolute power tolerance for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | f ≤ 3.0GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB 4.2GHz < f ≤ 6GHz Normal conditions ± 3.0 dB Extreme conditions ± 6.0 dB |  | 1.0 dB 1.0 dB 2.0 dB 2.0 dB |  | Formula: Upper limit + TT, Lower limit – TT Normal conditions ± 10.0 dB Extreme conditions ± 13.0 dB Normal conditions ± 5.0 dB Extreme conditions ± 8.0 dB |  |
| 6.3.5G.3 Power Control Absolute power tolerance for V2X Communication / Intra-band contiguous multi-carrier operation |  | 4.2GHz < f ≤ 6GHz Normal conditions ± 3.0 dB Extreme conditions ± 6.0 dB |  | 2.0 dB 2.0 dB |  | Formula: Upper limit + TT, Lower limit – TT Normal conditions ± 5.0 dB Extreme conditions ± 8.0 dB |  |
| 6.5.1 Frequency Error |  | Modulated carrier, f ≤ 4.2GHz Within 0.1 ppm compared to the received carrier frequency f ≤ 3.0GHz DL power: Refsens 3.0GHz < f ≤ 4.2GHz DL power: Refsens |  | 15 Hz 0.7 dB 1.0 dB |  | Formula: Modulated carrier frequency: Upper limit + TT, Lower limit – TT DL power: Refsens + TT Modulated carrier frequency error = (0.1 ppm + 15 Hz) Refsens +0.7dB Refsens +1.0dB |  |
| 6.5.1A.1 Frequency error for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.5.1 |  | Same as 6.5.1 |  | Same as 6.5.1 |  |
| 6.5.1A.2 Frequency Error for CA (inter-band DL CA and UL CA) |  | Same as 6.5.1 per CC |  | Same as 6.5.1 per CC |  | Same as 6.5.1 per CC |  |
| 6.5.1A.3 Frequency Error for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.5.1 per CC |  | Same as 6.5.1 per CC |  | Same as 6.5.1 per CC |  |
| 6.5.1A.5 Frequency error for CA (4UL CA) |  | Same as 6.5.1 per CC |  | Same as 6.5.1 per CC |  | Same as 6.5.1 per CC |  |
| 6.5.1B Frequency Error for UL-MIMO |  | Same as 6.5.1 |  | Same as 6.5.1 |  | Same as 6.5.1 |  |
| 6.5.1D.1 Frequency Error for ProSe Direct Discovery |  | Modulated carrier, f ≤ 4.2GHz Within 0.1 ppm compared to the received carrier frequency f ≤ 3.0GHz Sidelink power: Refsens |  | 15 Hz 0.7 dB |  | Formula: Modulated carrier frequency: Upper limit + TT, Lower limit – TT Sidelink power: Refsens + TT Modulated carrier frequency error = (0.1 ppm + 15 Hz) Refsens +0.7dB |  |
| 6.5.1D.2 Frequency Error for ProSe Direct Communication |  | Same as 6.5.1D.1 |  | Same as 6.5.1D.1 |  | Same as 6.5.1D.1 |  |
| 6.5.1E Frequency Error for UE category 0 |  | Same as 6.5.1 |  | Same as 6.5.1 |  | Same as 6.5.1 |  |
| 6.5.1EA Frequency Error for UE category M1 |  | Same as 6.5.1 |  | Same as 6.5.1 |  | Same as 6.5.1 |  |
| 6.5.1EB Frequency Error for UE category 1bis |  | Same as 6.5.1 |  | Same as 6.5.1 |  | Same as 6.5.1 |  |
| 6.5.1EC Frequency Error for UE category M2 |  | Same as 6.5.1 |  | Same as 6.5.1 |  | Same as 6.5.1 |  |
| 6.5.1EC_1 Frequency Error for UE category M2 (CE Mode B) |  | Same as 6.5.1 |  | Same as 6.5.1 |  | Same as 6.5.1 |  |
| 6.5.1F Frequency Error for category NB1 and NB2 |  | Modulated carrier, f ≤ 3.0GHz Within 0.1 ppm compared to the received carrier frequency f ≤ 3.0GHz DL power: Refsens 3.0GHz < f ≤ 4.2GHz DL power: Refsens |  | 15 Hz 0.7 dB 1.0 dB |  | Formula: Modulated carrier frequency: Upper limit + TT, Lower limit – TT DL power: Refsens + TT Modulated carrier frequency error = (0.1 ppm + 15 Hz) Refsens +0.7dB Refsens +1.0dB |  |
| 6.5.1G.1 Frequency Error for V2X Communication / Non-concurrent with E-UTRA uplink transmission |  | Modulated carrier, f ≤ 6.0GHz Within 0.1 ppm compared to the received carrier frequency |  | 36 Hz |  | Formula: Modulated carrier frequency: Upper limit + TT, Lower limit – TT DL power: Refsens + TT Modulated carrier frequency error = (0.1 ppm + 36 Hz) |  |
| 6.5.1G.2 Frequency error for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Modulated carrier, f ≤ 3.0GHz Within 0.1 ppm compared to the received carrier frequency f ≤ 3.0GHz DL power: Refsens Modulated carrier, 4.2GHz <f ≤ 6.0GHz Within 0.1 ppm compared to the received carrier frequency 4.2GHz <f ≤ 6.0GHz DL power: Refsens |  | 15 Hz 0.7 dB 36 Hz 1.5 dB |  | Formula: Modulated carrier frequency: Upper limit + TT, Lower limit – TT DL power: Refsens + TT Modulated carrier frequency error = (0.1 ppm + 15 Hz) Refsens +0.7 dB Modulated carrier frequency error = (0.1 ppm + 36 Hz) Refsens +1.5 dB |  |
| 6.5.2.1 Error Vector Magnitude |  | EVM limit: BPSK :17.5 % QPSK: 17.5 % 16QAM: 12.5 % |  | 0% |  | Formula: Minimum Requirement + TT |  |
| 6.5.2.1_1 Error Vector Magnitude (EVM) for UL 64QAM |  | EVM limit: 64QAM: 8.0 % |  | 0% |  | Formula: Minimum Requirement + TT |  |
| 6.5.2.1_2 Error Vector Magnitude (EVM) for UL 256QAM |  | EVM limit: 256QAM: 3.5 % |  | 0.3%, 15dBm < PUL 0.8%, -25dBm < PUL ≤ 15dBm, 1.1%, -30dBm ≤ PUL ≤ -25dBm |  | Formula: Minimum Requirement + TT |  |
| 6.5.2.1A PUSCH-EVM with exclusion period |  | EVM limit: QPSK: 17.5 % 16QAM: 12.5 % |  | 0% |  | Formula: Minimum Requirement + TT |  |
| 6.5.2A.1.1 Error Vector Magnitude (EVM) for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  |
| 6.5.2A.1.1_1 EVM for CA (intra-band contiguous DL CA and UL CA) with UL 64QAM |  | Same as 6.5.2.1_1 |  | Same as 6.5.2.1_1 |  | Same as 6.5.2.1_1 |  |
| 6.5.2A.1.1_2 EVM for CA (intra-band contiguous DL CA and UL CA) with UL 256QAM |  | EVM limit: 256QAM: 3.5 % |  | Same as 6.5.2.1_2 |  | Formula: Minimum Requirement + TT |  |
| 6.5.2A.1.2 Error Vector Magnitude (EVM) for CA (inter-band DL CA and UL CA) |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  |
| 6.5.2A.1.2_1 EVM for CA (inter-band DL CA and UL CA) with UL 64QAM |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  |
| 6.5.2A.1.2_2 Error Vector Magnitude (EVM) for CA (inter-band DL CA and UL CA) for UL 256QAM |  | EVM limit: 256QAM: 3.5 % |  | Same as 6.5.2.1_2 |  | Formula: Minimum Requirement + TT |  |
| 6.5.2A.1.3 Error Vector Magnitude (EVM) for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  |
| 6.5.2A.1.3_1 EVM for CA (intra-band non-contiguous DL CA and UL CA) with UL 64QAM |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  |
| 6.5.2A.1.3_2 Error Vector Magnitude (EVM) for CA (intra-band non-contiguous DL CA and UL CA) for UL 256QAM |  | EVM limit: 256QAM: 3.5 % |  | Same as 6.5.2.1_2 |  | Formula: Minimum Requirement + TT |  |
| 6.5.2A.1.4 Error Vector Magnitude (EVM) for CA (3UL CA) |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  |
| 6.5.2A.1.5 Error Vector Magnitude (EVM) for CA (4UL CA) |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  |
| 6.5.2B.1 Error Vector Magnitude (EVM) for UL- MIMO |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.5.2.1E.1 Error Vector Magnitude for UE category 0 |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  |
| 6.5.2.1E.2 PUSCH-EVM with exclusion period for UE category 0 |  | Same as 6.5.2.1A |  | Same as 6.5.2.1A |  |  |  |
| 6.5.2.1EA.1 Error Vector Magnitude for UE category M1 |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  |
| 6.5.2.1EA.2 PUSCH-EVM with exclusion period for UE category M1 |  | Same as 6.5.2.1A |  | Same as 6.5.2.1A |  |  |  |
| 6.5.2.1EB Error Vector Magnitude (EVM) for UE category 1bis |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  |
| 6.5.2.1EC.1 Error Vector Magnitude for UE category M2 |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  |
| 6.5.2.1EC.2 PUSCH-EVM with exclusion period for UE category M2 |  | Same as 6.5.2.1A |  | Same as 6.5.2.1A |  | Same as 6.5.2.1A |  |
| 6.5.2.1F.1 Error Vector Magnitude (EVM) for UE category NB1 and NB2 |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  |
| 6.5.2.1G.1 Error Vector Magnitude (EVM) for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | EVM limit: QPSK: 17.5 % 16QAM: 12.5 % |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  |
| 6.5.2.1G.2 Error Vector Magnitude (EVM) for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | EVM limit: QPSK: 17.5 % 16QAM: 12.5 % |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  |
| 6.5.2.1G.3 Error Vector Magnitude (EVM) for V2X Communication / Intra-band contiguous multi-carrier operation |  | EVM limit: QPSK: 17.5 % 16QAM: 12.5 % |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  |
| 6.5.2.1_s Error Vector Magnitude (EVM) for subslot/slot TTI |  | Same as 6.5.2 |  | Same as 6.5.2 |  | Same as 6.5.2 |  |
| 6.5.2.2 Carrier leakage |  | For Output power >0 dBm -25dBc For -30 dBm ≤ Output power ≤0 dBm -20dBc For -40 dBm ≤ Output power < -30 dBm -10dBc |  | 0.8dB |  | Formula: Minimum Requirement + TT |  |
| 6.5.2.2F Carrier leakage for UE category NB1 and NB2 |  | Same as 6.5.2.2 |  | Same as 6.5.2.2 |  | Same as 6.5.2.2 |  |
| 6.5.2.2G.1 Carrier leakage for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | For Output power >0 dBm -25dBc For -30 dBm ≤ Output power ≤0 dBm -20dBc |  | Same as 6.5.2.2 |  | Same as 6.5.2.2 |  |
| 6.5.2.2G.2 Carrier leakage for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission |  | For E-UTRA carrier, same as 6.5.2.2 For V2X carrier, same as 6.5.2.2G.1 |  | Same as 6.5.2.2 |  | Same as 6.5.2.2 |  |
| 6.5.2.2G.3 Carrier leakage for V2X Communication / Intra-band contiguous MCC operation |  | For Output power >0 dBm -25dBc For -30 dBm ≤ Output power ≤0 dBm -20dBc |  | Same as 6.5.2.2 |  | Same as 6.5.2.2 |  |
| 6.5.2.3 In-band emissions for non allocated RB |  | For general emissions: within the subPRB allocation -18-5∙¦∆SubG-1)/LSCG For general emissions for any non-allocated RB: For IQ image: -25dB For Carrier leakage: Output power >0 dBm -25dBc -30 dBm ≤ Output power ≤0 dBm -20dBc -40 dBm ≤ Output power < -30 dBm -10dBc For each evaluated RB, the test requirement is calculated as the higher of PRB – 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) |  | 0.8dB |  | Formula: Minimum Requirement + TT |  |
| 6.5.2.4 EVM equalizer Spectrum flatness |  | Normal conditions. If (F-FUL_low ≥ [3MHz])&(FUL_high-F≥ [3MHz]) 4 dB else 8 dB maximum coefficient in Range 1 - the minimum coefficient in Range 2 5 dB the maximum coefficient in Range 2 - the minimum coefficient in Range 7 dB Extreme conditions: If (F-FUL_low ≥ [5MHz])&(FUL_high-F≥ [5MHz]) 4 dB else 12 dB maximum coefficient in Range 1 - the minimum coefficient in Range 2 6 dB the maximum coefficient in Range 2 - the minimum coefficient in Range 10 dB |  | 1.4dB |  | Formula: Minimum Requirement + TT |  |
| 6.5.2A.2.1 Carrier leakage for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.5.2.2 |  | Same as 6.5.2.2 |  | Same as 6.5.2.2 |  |
| 6.5.2A.2.2 Carrier leakage for CA (inter-band DL CA and UL CA) |  | For Output power >0 dBm -25dBc For -30 dBm ≤ Output power ≤0 dBm -20dBc For -40 dBm ≤ Output power < -30 dBm -10dBc |  | 0.8dB |  | Formula: Minimum Requirement + TT |  |
| 6.5.2A.2.3 Carrier leakage for CA (intra-band non-contiguous DL CA and UL CA) |  | For Output power >0 dBm -25dBc For -30 dBm ≤ Output power ≤0 dBm -20dBc For -40 dBm ≤ Output power < -30 dBm -10dBc |  | 0.8dB |  | Formula: Minimum Requirement + TT |  |
| 6.5.2A.2.4 Carrier leakage for CA (3UL CA) |  | Same as 6.5.2.2 |  | Same as 6.5.2.2 |  | Same as 6.5.2.2 |  |
| 6.5.2A.2.5 Carrier leakage for CA (4UL CA) |  | Same as 6.5.2.2 |  | Same as 6.5.2.2 |  | Same as 6.5.2.2 |  |
| 6.5.2A.3.1 In-band emissions for non allocated RB for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.5.2.3 |  | Same as 6.5.2.3 |  | Same as 6.5.2.3 |  |
| 6.5.2A.3.2 In-band emissions for non allocated RB for CA (inter-band DL CA and UL CA) |  | Same as 6.5.2.3 |  | Same as 6.5.2.3 |  | Same as 6.5.2.3 |  |
| 6.5.2A.3.3 In-band emissions for non allocated RB for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.5.2.3 |  | Same as 6.5.2.3 |  | Same as 6.5.2.3 |  |
| 6.5.2A.3.4 In-band emissions for non allocated RB for CA (3UL CA) |  | Same as 6.5.2.3 |  | Same as 6.5.2.3 |  | Same as 6.5.2.3 |  |
| 6.5.2A.3.5 In-band emissions for non allocated RB for CA (4UL CA) |  | Same as 6.5.2.3 |  | Same as 6.5.2.3 |  | Same as 6.5.2.3 |  |
| 6.5.2B.1 Error vector magnitude (EVM) for UL-MIMO |  | TBD |  | TBD |  | TBD |  |
| 6.5.2B.2 Carrier leakage for UL-MIMO |  | Same as 6.5.2.2 |  | Same as 6.5.2.2 |  | Same as 6.5.2.2 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.5.2B.3 In-band emissions for non allocated RB for UL-MIMO |  | Same as 6.5.2.3 |  | Same as 6.5.2.3 |  | Same as 6.5.2.3 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.5.2B.4 EVM equalizer spectrum flatness for UL-MIMO |  | Same as 6.5.2.4 |  | Same as 6.5.2.4 |  | Same as 6.5.2.4 |  |
| 6.5.2.2E Carrier Leakage for UE category 0 |  | Same as 6.5.2.2 |  | Same as 6.5.2.2 |  | Same as 6.5.2.2 |  |
| 6.5.2.2EA Carrier Leakage for UE category M1 |  | Same as 6.5.2.2 |  | Same as 6.5.2.2 |  | Same as 6.5.2.2 |  |
| 6.5.2.2EB Carrier leakage for UE category 1bis |  | Same as 6.5.2.2 |  | Same as 6.5.2.2 |  | Same as 6.5.2.2 |  |
| 6.5.2.2EC Carrier leakage for UE category M2 |  | Same as 6.5.2.2 |  | Same as 6.5.2.2 |  | Same as 6.5.2.2 |  |
| 6.5.2.3E In-band emissions for non allocated RB for UE category 0 |  | Same as 6.5.2.3 |  | Same as 6.5.2.3 |  | Same as 6.5.2.3 |  |
| 6.5.2.3EA In-band emissions for non allocated RB for UE category M1 |  | Same as 6.5.2.3 |  | Same as 6.5.2.3 |  | Same as 6.5.2.3 |  |
| 6.5.2.3EB In-band emissions for non allocated RB for UE category 1bis |  | Same as 6.5.2.3 |  | Same as 6.5.2.3 |  | Same as 6.5.2.3 |  |
| 6.5.2.3EC In-band emissions for non allocated RB for UE category M2 |  | Same as 6.5.2.3 |  | Same as 6.5.2.3 |  | Same as 6.5.2.3 |  |
| 6.5.2.3F In-band emissions for non allocated RB for UE category NB1 and NB2 |  | For general emissions: For IQ image: -25dB For Carrier leakage: Output power >0 dBm -25dBc -30 dBm ≤ Output power ≤0 dBm -20dBc -40 dBm ≤ Output power < -30 dBm -10dBc For each evaluated tone, the test requirement is calculated as the higher of Ptone – 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) |  | Same as 6.5.2.3 |  | Same as 6.5.2.3 |  |
| 6.5.2.3G.1 In-band emissions for non-allocated RB for V2X Communication |  | For general emissions: For IQ image: -25dB For Carrier leakage: Output power >0 dBm -25dBc -30 dBm ≤ Output power ≤0 dBm -20dBc For each evaluated RB, the test requirement is calculated as the higher of PRB – 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) Sidelink Power Window >= 0dBm, (-30 dBm, 0 dBm) E-UTRA Uplink Power Window >= 0dBm, (-30 dBm, 0 dBm) |  | Emission: 0.8dB Sidelink Power: Absolute Transmitted Power tolerance: ±3dB referting to the TS 36.101 [2] clasue 6.3.5G.1 Wanted signal uncertainty: ±1.3 dB 4.2GHz < f ≤ 6.0GHz, for UE powers >= 0dBm ±1.5 dB, 4.2GHz < f ≤ 6.0GHz, for UE powers >= -40dBm Uplink power Relative Transmitted Power tolerance: ±2.5dB referting to the TS 36.101 [2] clasue 6.3.5.2 Test system power measurement uncertainty: ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.3 dB, 4.2GHz < f ≤ 6GHz |  | Formula for emission: Minimum Requirement + TT Sidelink power measurement window: Bottom of the window: the Configured sidelink power - TT of sidelink power Top of the window: the Configured sidelink power + TT of sidelink power TT of sidelink power comprises 2 quantities: 1. Absolute Transmitted Power tolerance :±3dB 2. Test system power measurement uncertainty: ±1.3 dB 4.2GHz < f ≤ 6.0GHz for UE powers>= 0dBm. ±1.5 dB 4.2GHz < f ≤ 6.0GHz, for UE powers >= -40dBm Items 1 and 2 are added arithmetically: TT of sidelink power: (3dB+1.3dB) =4.3dB, 4.2GHz < f ≤ 6.0GH for UE powers >= 0 dBm: TT of sidelink power: (3dB+1.5dB) =4.5dB, 4.2GHz < f ≤ 6.0GH for powers >= -40 dBm: Ensure the bottom of the sidelink power measurement window is aligned with the bottom of the sidelink power window of the corresponding IBE requirement, and the top of the sidelink power measurement window is below the top of sidelink power window. Uplink power measurement window: Bottom of the window: the Configured sidelink power - TT of sidelink power Top of the window: the Configured sidelink power + TT of sidelink power TT of uplink power comprises 2 quantities: 1. Relative Transmitted Power tolerance :±2.5dB 2. Test system power measurement uncertainty: ±0.7 dB for f ≤ 3.0GHz. ±1 dB for 3.0GHz < f ≤ 4.2GHz Items 1 and 2 are added arithmetically: For f ≤ 3.0GHz TT of uplink power: (2.5dB+0.7dB) =3.2dB For 3.0GHz < f ≤ 4.2GHz TT of uplink power: (2.5dB+1dB) =3.5dB, Ensure the bottom of the uplink power measurement window is aligned with the bottom of the uplink power window of the corresponding IBE requirement, and the top of the uplink power measurement window is below the top of uplink power window. |  |
| 6.5.2.3G.2 In-band emissions for non-allocated RB for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | same as 6.5.2.3 for E-UTRA uplink same as 6.5.2.3G.1 for E-UTRA V2X sidelink |  | same as 6.5.2.3 for E-UTRA uplink same as 6.5.2.3G.1 for E-UTRA V2X sidelink |  | same as 6.5.2.3 for E-UTRA uplink same as 6.5.2.3G.1 for E-UTRA V2X sidelink |  |
| 6.5.2.3G.3 In-band emissions for non-allocated RB for V2X Communication / Intra-band contiguous MCC operation |  | same as 6.5.2.3G.1 for each cc of E-UTRA V2X sidelink |  | same as 6.5.2.3G.1 for each cc of E-UTRA V2X sidelink |  | same as 6.5.2.3G.1 for each cc of E-UTRA V2X sidelink |  |
| 6.5.2.4E EVM equalizer spectrum flatness for UE category 0 |  | Same as 6.5.2.4 |  | Same as 6.5.2.4 |  | Same as 6.5.2.4 |  |
| 6.5.2.4EA EVM equalizer spectrum flatness for UE category M1 |  | Same as 6.5.2.4 |  | Same as 6.5.2.4 |  | Same as 6.5.2.4 |  |
| 6.5.2.4EB EVM equalizer spectrum flatness for UE category 1bis |  | Same as 6.5.2.4 |  | Same as 6.5.2.4 |  | Same as 6.5.2.4 |  |
| 6.5.2.4EC EVM equalizer spectrum flatness for UE category M2 |  | Same as 6.5.2.4 |  | Same as 6.5.2.4 |  | Same as 6.5.2.4 |  |
| 6.5.2.4G.1 EVM equalizer spectrum flatness for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Same as 6.5.2.4 |  | Same as 6.5.2.4 |  | Same as 6.5.2.4 |  |
| 6.5.2.4G.3 EVM equalizer spectrum flatness for V2X Communication / Intra-band contiguous multi-carrier operation uplink transmissions |  | Same as 6.5.2.4 |  | Same as 6.5.2.4 |  | Same as 6.5.2.4 |  |
| 6.6.1 Occupied bandwidth |  | For 1.4 MHz channel bandwidth: Occupied channel bandwidth = 1.4 MHz For 3.0 MHz channel bandwidth: Occupied channel bandwidth = 3.0 MHz For 5 MHz channel bandwidth: Occupied channel bandwidth = 5 MHz For 10 MHz channel bandwidth: Occupied channel bandwidth = 10 MHz For 15 MHz channel bandwidth: Occupied channel bandwidth = 15 MHz For 20 MHz channel bandwidth: Occupied channel bandwidth = 20 MHz |  | 0kHz |  | Formula: Minimum Requirement + TT |  |
| 6.6.1A.1 Occupied bandwidth for CA (intra-band contiguous DL CA and UL CA) |  | For 25(20+5) MHz channel bandwidth: Occupied channel bandwidth = 24.95 MHz For 30(20+10) MHz channel bandwidth: Occupied channel bandwidth = 29.90 MHz For 30(15+15) MHz channel bandwidth: Occupied channel bandwidth = 30.00 MHz For 35(20+15)MHz channel bandwidth: Occupied channel bandwidth = 34.85 MHz For 40(20+20) MHz channel bandwidth: Occupied channel bandwidth = 39.80 MHz |  | Same as 6.6.1 |  | Same as 6.6.1 |  |
| 6.6.1A.2 Occupied bandwidth for CA (inter-band DL CA and UL CA) |  | Same as 6.6.1 per CC |  | Same as 6.6.1 per CC |  | Same as 6.6.1 per CC |  |
| 6.6.1A.3 |  | Same as 6.6.1 for each CC |  | Same as 6.6.1 |  | Same as 6.6.1 for each CC |  |
| 6.6.1A.5 Occupied bandwidth for CA (4UL CA) |  | Same as 6.6.1 for each CC |  | Same as 6.6.1 |  | Same as 6.6.1 for each CC |  |
| 6.6.1B Occupied bandwidth for UL-MIMO |  | Same as 6.6.1 |  | Same as 6.6.1 |  | Same as 6.6.1 |  |
| 6.6.1E Occupied bandwidth for UE category 0 |  | Same as 6.6.1 |  | Same as 6.6.1 |  | Same as 6.6.1 |  |
| 6.6.1EA Occupied bandwidth for UE category M1 |  | Same as 6.6.1 |  | Same as 6.6.1 |  | Same as 6.6.1 |  |
| 6.6.1EB Occupied bandwidth for UE category 1bis |  | Same as 6.6.1 |  | Same as 6.6.1 |  | Same as 6.6.1 |  |
| 6.6.1EC Occupied bandwidth for UE category M2 |  | Same as 6.6.1 |  | Same as 6.6.1 |  | Same as 6.6.1 |  |
| 6.6.1F Occupied bandwidth for UE category NB1 |  | Occupied channel bandwidth = 200KHz |  | 0kHz |  | Formula: Minimum Requirement + TT |  |
| 6.6.1G.1 Occupied bandwidth for V2X Communication / Non-concurrent with E-UTRA uplink transmission |  | Same as 6.6.1 |  | Same as 6.6.1 |  | Same as 6.6.1 |  |
| 6.6.1G.2 Occupied bandwidth for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Same as 6.6.1 |  | Same as 6.6.1 |  | Same as 6.6.1 |  |
| 6.6.1G.3 Occupied bandwidth for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 6.6.1 |  | Same as 6.6.1 |  | Same as 6.6.1 |  |
| 6.6.2.1 Spectrum Emission Mask |  | For 1.4 MHz BW: -10 dBm / 30kHz -25dBm to -10dBm / 1MHz For 3 MHz BW: -13 dBm / 30kHz -25dBm to -10dBm / 1MHz For 5 MHz BW: -15dBm / 30kHz -25dBm to -10dBm / 1MHz For 10 MHz BW: -18dBm / 30kHz -25dBm to -10dBm / 1MHz For 15 MHz BW: -20dBm / 30kHz -25dBm to -10dBm / 1MHz For 20 MHz BW: -21dBm / 30kHz -25dBm to -10dBm / 1MHz |  | All cases: f ≤ 3.0GHz 1.5dB 3.0GHz < f ≤ 4.2GHz 1.8dB |  | Formula: Minimum Requirement + TT Note: The Test Tolerance would be 0dB for ΔfOOB ≥ 2 x Channel Bandwidth, but taking into account the filter position, the Test requirements specified all have ΔfOOB < 2 x Channel Bandwidth |  |
| 6.6.2.1_1 Spectrum Emission Mask for Multi-Cluster PUSCH |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |
| 6.6.2.1A.1 Spectrum emission mask for CA (intra-band contiguous DL CA and UL CA) |  | For 9.8MHz BW: -18 dBm / 30kHz -25 dBm to -10 dBm / 1MHz For 14.95MHz BW: -20 dBm / 30kHz -25 dBm to -10 dBm / 1MHz For 19.8MHz BW: -21 dBm / 30kHz -25 dBm to -10 dBm / 1MHz For 19.9MHz: -21 dBm / 30kHz -25 dBm to -10 dBm / 1MHz For 24.75MHz BW: -22 dBm / 30kHz -25 dBm to -10 dBm / 1MHz For 24.95 MHz BW: -22 dBm / 30kHz -25 dBm to - 10 dBm / 1 MHz For 29.9 MHz BW: -22.5 dBm / 30kHz -25dBm to -10dBm / 1MHz For 30 MHz BW: -22.5 dBm / 30kHz -25dBm to -10dBm / 1MHz For 34.85 MHz BW: -23.5dBm / 30kHz -25dBm to -10dBm / 1MHz For 39.8 MHz BW: -24dBm / 30kHz -25dBm to -10dBm / 1MHz |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |
| 6.6.2.1A.2 Spectrum emission mask for CA (inter-band DL CA and UL CA) |  | Same as 6.6.2.1 per CC |  | Same as 6.6.2.1 per CC |  | Same as 6.6.2.1 per CC |  |
| 6.6.2.1A.3 Spectrum Emission Mask for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.6.2.1 for each CC |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 for each CC |  |
| 6.6.2.1A.5 Additional Spectrum Emission Mask for CA (4UL) |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |
| 6.6.2.1B Spectrum Emission Mask for UL-MIMO |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |
| 6.6.2.1E Spectrum Emission Mask for UE category 0 |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |
| 6.6.2.1EA Spectrum Emission Mask for UE category M1 |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |
| 6.6.2.1EB Spectrum Emission Mask for UE category 1bis |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |
| 6.6.2.1EC Spectrum Emission Mask for UE category M2 |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |
| 6.6.2.1F Spectrum Emission Mask for UE category NB1 |  | For all frequencies -35dBm to 26dBm / 30kHz For frequencies that are Foffset away from edge of NB1 channel edge as defined in Table 6.6.2.1F.3-2, same as 6.6.2.1 |  | All cases: f ≤ 3.0GHz 1.5dB |  | For all frequencies interpolated Minimum Requirement + TT |  |
| 6.6.2.1G.1 Spectrum Emission Mask for V2X Communication Non-concurrent with E-UTRA uplink transmissions |  | Bandwidth: 10MHz -20dBm /30kHz -25 ~ -10 / 1 MHz Bandwidth: 20MHz -21dBm /30kHz -25 ~ -10 / 1 MHz |  | ±2.0 dB, 4.2GHz < f ≤ 6GHz |  | Formula: Minimum Requirement + TT Note: The Test Tolerance would be 0dB for ΔfOOB ≥ 2 x Channel Bandwidth, but taking into account the filter position, the Test requirements specified all have ΔfOOB < 2 x Channel Bandwidth |  |
| 6.6.2.1G.2 Spectrum Emission Mask for V2X Communication /Sidelink simultaneous with E-UTRA uplink transmissions |  | Same as 6.6.2.1G.1 for sidelink Same as 6.6.2.1G.1 for E-UTRA uplink |  | Same as 6.6.2.1G.1 |  | Same as 6.6.2.1G.1 |  |
| 6.6.2.1G.3 Spectrum Emission Mask for V2X Communication / Intra-band contiguous MCC operation |  | Same as 6.6.2.1G.1 |  | Same as 6.6.2.1G.1 |  | Same as 6.6.2.1G.1 |  |
| 6.6.2.2 Additional Spectrum Emission Mask |  | For 1.4 MHz BW: NS_03, NS_04 -10 dBm / 30 kHz -25 dBm to -13 dBm / 1MHz NS_06 or NS_07 -13 dBm / 30 kHz -13 dBm / 100 kHz -25 dBm to -13 dBm / 1MHz For 3 MHz BW: NS_03, NS_04 -13 dBm / 30 kHz -25 dBm to -13 dBm / 1 MHz NS_06 or NS_07 -13 dBm / 30 kHz -13 dBm / 100kHz -25 dBm to -13 dBm / 1 MHz For 5 MHz BW: NS_03, NS_04 -15 dBm / 30 kHz -25 dBm to -13 dBm / 1 MHz NS_06 or NS_07 -15 dBm / 30 kHz -13 dBm / 100 kHz -25 dBm to -13 dBm / 1 MHz For 10 MHz BW: NS_03, NS_04, -18 dBm / 30 kHz -25 dBm to - 13dBm / 1 MHz NS_06 or NS_07 -18 dBm / 30 kHz -13 dBm / 100 kHz -25 dBm to - 13dBm / 1 MHz For 15 MHz BW: NS_03, NS_04 -20 dBm / 30kHz -25 dBm to -13 dBm / 1 MHz For 20 MHz BW: NS_03, NS_04 -21 dBm / 30 kHz -25 dBm to -13 dBm / 1 MHz |  | All cases: f ≤ 3.0GHz 1.5dB 3.0GHz < f ≤ 4.2GHz 1.8dB 4.2GHz < f ≤ 6.0GHz ±2.0 dB |  | Formula: Minimum Requirement + TT Note: The Test Tolerance would be 0dB for ΔfOOB ≥ 2 x Channel Bandwidth, but taking into account the filter position, the Test requirements specified all have ΔfOOB < 2 x Channel Bandwidth |  |
| 6.6.2.2_1 Additional Spectrum Emission Mask for UL 64QAM |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  |
| 6.6.2.2_2 Additional Spectrum Emission Mask for UL 256QAM |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  |
| 6.6.2.2A-1 Additional Spectrum Emission Mask for CA (intra-band contiguous DL CA and UL CA) |  | For network signalled value CA_NS_04: -21dBm/30kHz to -22.5dBm/kHz, BW-dependent, for 0-1MHz -13dBm/1MHz for 1MHz-5.5MHz -25dBm/1MHz beyond 5.5MHz |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  |
| 6.6.2.2A.1_1 Additional Spectrum Emission Mask for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM |  | Same as 6.6.2.2A.1 |  | Same as 6.6.2.2A.1 |  | Same as 6.6.2.2A.1 |  |
| 6.6.2.2A.1_2 Additional Spectrum Emission Mask for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM |  | Same as 6.6.2.2A.1 |  | Same as 6.6.2.2A.1 |  | Same as 6.6.2.2A.1 |  |
| 6.6.2.2A.2 Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA) |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  |
| 6.6.2.2A.2_1 Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA) for UL 64QAM |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  |
| 6.6.2.2A.2_2 Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA) for UL 256QAM |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  |
| 6.6.2.2A.3 Additional Spectrum Emission Mask for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  |
| 6.6.2.2A.5 Additional Spectrum Emission Mask for CA (4UL CA) |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  |
| 6.6.2.2B Additional Spectrum Emission Mask for UL-MIMO |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  |
| 6.6.2.2E Additional Spectrum Emission Mask for UE category 0 |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  |
| 6.6.2.2EA Additional Spectrum Emission Mask for UE category M1 |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  |
| 6.6.2.2EB Additional Spectrum Emission Mask for UE category 1bis |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  |
| 6.6.2.2EC Additional Spectrum Emission Mask for UE category M2 |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  |
| 6.6.2.2G.1 Additional Spectrum Emission Mask for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | ΔfOOB(MHz) 0-0.5 [-13-12∆fOOBMHz] ΔfOOB(MHz) 0.5-5 [-19-169∆fOOBMHz-0.5] ΔfOOB(MHz) 5-10 [-27-2∆fOOBMHz-5.0{ QUOTE -13-12∆fOOBMHz}] |  | Same as 6.6.2.2 |  | Formula: Minimum Requirement + TT |  |
| 6.6.2.2G.2 Additional Spectrum Emission Mask for V2X Communication / Sidelink simultaneous with E-UTRA uplink transmissions |  | same as 6.6.2.2 for E-UTRA uplink same as 6.6.2.2G.1 for sidelink |  | same as 6.6.2.2 for E-UTRA uplink same as 6.6.2.2G.1 for sidelink |  | Formula: Minimum Requirement + TT |  |
| 6.6.2.3 Adjacent Channel Leakage power Ratio |  | If the adjacent channel power is greater than –50 dBm then the ACLR shall be higher than the values specified below. E-UTRA ACLR: 30 dB UTRA ACLR: 33 dB for UTRA ACLR 1 36 dB for UTRA ACLR 2 |  | 0 dB 0.8 dB 0.8 dB 0.8 dB |  | Formula: ACLR Minimum Requirement + TT Formula: ACLR Minimum Requirement - TT E-UTRA ACLR: 29.2 dB UTRA ACLR: 32.2 dB for UTRA ACLR 1 35.2 dB for UTRA ACLR 2 |  |
| 6.6.2.3_1 Adjacent Channel Leakage power Ratio for HPUE |  | If the adjacent channel power is greater than –50 dBm then the ACLR shall be higher than the values specified below. E-UTRA ACLR for Power Class 1: 37 dB E-UTRA ACLR for Power Class 2: 31 dB |  | 0dB 0.8dB |  | Formula: ACLR Minimum Requirement + TT Formula: ACLR Minimum Requirement - TT E-UTRA ACLR for Power Class 1: 36.2 dB E-UTRA ACLR for Power Class 2: 30.2 dB |  |
| 6.6.2.3_2 Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |
| 6.6.2.3_3 Adjacent Channel Leakage power Ratio for UL 64QAM |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |
| 6.6.2.3_4 Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH with UL 64QAM |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |
| 6.6.2.3_5 Adjacent Channel Leakage power Ratio for UL 256QAM |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |
| 6.6.2.3_6 Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH with UL 256QAM |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |
| 6.6.2.3A.1 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) |  | If the adjacent channel power is greater than –50 dBm then the ACLR shall be higher than the values specified below. UTRA ACLR: 33 dB for UTRA ACLR 1 36 dB for UTRA ACLR 2 E-UTRA ACLR: 30 dB |  | 0 dB 0.8 dB 0.8 dB 0.8 dB |  | Formula: ACLR Minimum Requirement + TT Formula: ACLR Minimum Requirement - TT UTRA ACLR: 32.2 dB for UTRA ACLR 1 35.2 dB for UTRA ACLR 2 E-UTRA ACLR: 29.2 dB |  |
| 6.6.2.3A.1_1 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM |  | Same as 6.6.2.3A.1 |  | Same as 6.6.2.3A.1 |  | Same as 6.6.2.3A.1 |  |
| 6.6.2.3A.1_2 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM |  | Same as 6.6.2.3A.1 |  | Same as 6.6.2.3A.1 |  | Same as 6.6.2.3A.1 |  |
| 6.6.2.3A.2 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.6.2.3 per component carrier E-UTRA ACLR2: 40dB |  | Same as 6.6.2.3 per component carrier 1.3dB |  | Same as 6.6.2.3 per component carrier E-UTRA ACLR: 38.7 dB |  |
| 6.6.2.3A.2_1 Adjacent Channel Leakage power Ratio for CA (inter-band DL CA and UL CA) for UL 64QAM |  | Same as 6.6.2.3 per component carrier |  | Same as 6.6.2.3 per component carrier |  | Same as 6.6.2.3 per component carrier |  |
| 6.6.2.3A.2_2 Adjacent Channel Leakage power Ratio for CA (inter-band DL CA and UL CA) for UL 256QAM |  | Same as 6.6.2.3A.2 |  | Same as 6.6.2.3A.2 |  | Same as 6.6.2.3A.2 |  |
| 6.6.2.3A.3 Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |
| 6.6.2.3A.3_1 Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous DL CA and UL CA) for UL 64QAM |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |
| 6.6.2.3A.3_2 Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous DL CA and UL CA) for UL 256QAM |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |
| 6.6.2.3A.4.1 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous 3DL CA and 3UL CA) |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |
| 6.6.2.3A.4.2 Adjacent Channel Leakage power Ratio for CA (inter-band 3DL CA and 3UL CA) |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |
| 6.6.2.3A.4.3 Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous 3DL CA and 3UL CA) |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |
| 6.6.2.3A.5 Adjacent Channel Leakage power Ratio for CA (4UL CA) |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |
| 6.6.2.3B Adjacent Channel Leakage power Ratio for UL-MIMO |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |
| 6.6.2.3E Adjacent Channel Leakage power Ratio for UE category 0 |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |
| 6.6.2.3EA Adjacent Channel Leakage power Ratio for UE category M1 |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |
| 6.6.2.3EB Adjacent Channel Leakage power Ratio for UE category 1bis |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |
| 6.6.2.3EC Adjacent Channel Leakage power Ratio for UE category M2 |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |
| 6.6.2.3F Adjacent Channel Leakage power Ratio for UE category NB1 |  | If the adjacent channel power is greater than –50 dBm then the ACLR shall be higher than the values specified below. GSM ACLR: 20 dB UTRA ACLR: 37 dB for UTRA ACLR |  | 0 dB 0.8 dB 0.8 dB |  | Formula: ACLR Minimum Requirement + TT Formula: ACLR Minimum Requirement - TT GSM ACLR: 19.2 dB UTRA ACLR: 36.2 dB for UTRA ACLR |  |
| 6.6.2.3G.1 Adjacent Channel Leakage power Ratio for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | If the adjacent channel power is greater than –50 dBm then the ACLR shall be higher than the values specified below. E-UTRA ACLR: 30dB for power class 3 UE 31dB for power class 2 UE |  | 0.8 dB 0.8 dB |  | Formula: ACLR Minimum Requirement + TT Formula: ACLR Minimum Requirement - TT E-UTRA ACLR: 29.2dB for power class 3 UE 30.2dB for power class 2 UE |  |
| 6.6.2.3G.2 Adjacent Channel Leakage power Ratio for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Same as 6.6.2.3G.1 |  | Same as 6.6.2.3G.1 |  | Same as 6.6.2.3G.1 |  |
| 6.6.2.3G.3 Adjacent Channel Leakage power Ratio for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 6.6.2.3G.1 |  | Same as 6.6.2.3G.1 |  | Same as 6.6.2.3G.1 |  |
| 6.6.2.4 Additional ACLR requirements |  | If the adjacent channel power is greater than –50 dBm then the ACLR shall be higher than the values specified below. E-UTRA ACLR: 43 dB for UTRA ACLR 2 |  | 0 dB 0.8 dB |  | Formula: ACLR Minimum Requirement + TT Formula: ACLR Minimum Requirement – TT E-UTRA ACLR: 42.2 dB for UTRA ACLR 2 |  |
| 6.6.3.1 Transmitter Spurious emissions |  | 9 kHz f < 150 kHz: -36dBm / 1kHz 150 kHz f < 30 MHz: -36dBm / 10kHz 30 MHz f < 1 GHz: -36dBm / 100kHz 1 GHz f < 12.75 GHz: -30dBm / 1MHz 12.75 GHz ≤ f < 19 GHz: -30dBm / 1MHz |  | 0 dB |  | Formula: Minimum Requirement + TT |  |
| 6.6.3.1_1Transmitter Spurious emissions for Multi-Cluster PUSCH |  | Same as 6.6.3.1 |  | Same as 6.6.3.1 |  | Formula: Minimum Requirement + TT |  |
| 6.6.3.1A.1 Transmitter Spurious emissions for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.6.3.1 |  | Same as 6.6.3.1 |  | Same as 6.6.3.1 |  |
| 6.6.3.1A.2 Transmitter Spurious emissions for CA (inter-band DL CA and UL CA) |  | Same as 6.6.3.1 per CC |  | Same as 6.6.3.1 per CC |  | Same as 6.6.3.1 per CC |  |
| 6.6.3.1A.3 Transmitter Spurious emissions for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.6.3.1 per CC |  | Same as 6.6.3.1 per CC |  | Same as 6.6.3.1 per CC |  |
| 6.6.3.1A.4.1 Transmitter Spurious emissions for CA (3UL CA) (intra-band contiguous 3DL CA and 3UL CA) |  | Same as 6.6.3.1 per CC |  | Same as 6.6.3.1 per CC |  | Same as 6.6.3.1 per CC |  |
| 6.6.3.1A.4.2 Transmitter Spurious emissions for CA (3UL CA) (inter-band 3DL CA and 3UL CA) |  | Same as 6.6.3.1 per CC |  | Same as 6.6.3.1 per CC |  | Same as 6.6.3.1 per CC |  |
| 6.6.3.1A.4.3 Transmitter Spurious emissions for CA (3UL CA) (intra-band non-contiguous 3DL CA and 3UL CA) |  | Same as 6.6.3.1 per CC |  | Same as 6.6.3.1 per CC |  | Same as 6.6.3.1 per CC |  |
| 6.6.3.1A.5 Transmitter Spurious emissions for CA (4UL CA) |  | Same as 6.6.3.1 per CC |  | Same as 6.6.3.1 per CC |  | Same as 6.6.3.1 per CC |  |
| 6.6.3B.1Transmitter Spurious emissions for UL-MIMO |  | Same as 6.6.3.1 |  | Same as 6.6.3.1 |  | Formula: Minimum Requirement + TT |  |
| 6.6.3E.1 Transmitter Spurious emissions for UE category 0 |  | Same as 6.6.3.1 |  | Same as 6.6.3.1 |  | Same as 6.6.3.1 |  |
| 6.6.3E.2 Spurious emission band UE co-existence for UE category 0 |  | Same as 6.6.3.2 |  | Same as 6.6.3.2 |  |  |  |
| 6.6.3E.3 Additional spurious emissions for UE category 0 |  | Same as 6.6.3.3 |  | Same as 6.6.3.3 |  | Same as 6.6.3.3 |  |
| 6.6.3EA.1 Transmitter Spurious emissions for UE category M1 |  | Same as 6.6.3.1 |  | Same as 6.6.3.1 |  | Same as 6.6.3.1 |  |
| 6.6.3EA.2 Spurious emission band UE co-existence for UE category M1 |  | Same as 6.6.3.2 |  | Same as 6.6.3.2 |  |  |  |
| 6.6.3EA.3 Additional spurious emissions for UE category M1 |  | Same as 6.6.3.3 |  | Same as 6.6.3.3 |  | Same as 6.6.3.3 |  |
| 6.6.3EB.1 Transmitter Spurious emissions for UE category 1bis |  | Same as 6.6.3.1 |  | Same as 6.6.3.1 |  | Same as 6.6.3.1 |  |
| 6.6.3EB.2 Spurious emission band UE co-existence for UE category 1bis |  | Same as 6.6.3.2 |  | Same as 6.6.3.2 |  | Same as 6.6.3.2 |  |
| 6.6.3EB.3 Additional spurious emissions for UE category 1bis |  | Same as 6.6.3.3 |  | Same as 6.6.3.3 |  | Same as 6.6.3.3 |  |
| 6.6.3EC.1 Transmitter Spurious emissions for UE category M2 |  | Same as 6.6.3.1 |  | Same as 6.6.3.1 |  | Same as 6.6.3.1 |  |
| 6.6.3EC.2 Spurious emission band UE co-existence for UE category M2 |  | Same as 6.6.3.2 |  | Same as 6.6.3.2 |  | Same as 6.6.3.2 |  |
| 6.6.3EC.3 Additional spurious emissions for UE category M2 |  | Same as 6.6.3.3 |  | Same as 6.6.3.3 |  | Same as 6.6.3.3 |  |
| 6.6.3F.1 Transmitter Spurious emissions for UE category NB1 |  | Same as 6.6.3.1 |  | Same as 6.6.3.1 |  | Same as 6.6.3.1 |  |
| 6.6.3G.1 Transmitter Spurious emissions for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | 9 kHz f < 150 kHz: -36dBm / 1kHz 150 kHz f < 30 MHz: -36dBm / 10kHz 30 MHz f < 1 GHz: -36dBm / 100kHz 1 GHz f < 12.75 GHz: -30dBm / 1MHz 12.75 GHz ≤ f < 26 GHz: -30dBm / 1MHz |  | 0 dB |  | Formula: Minimum Requirement + TT |  |
| 6.6.3G.1_1 Spurious emission band UE co-existence for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Same as 6.6.3G.1 |  | Same as 6.6.3G.1 |  | Formula: Minimum Requirement + TT |  |
| 6.6.3G.2 Spurious emission for V2X Communication / Sidelink simultaneous with E-UTRA uplink transmissions |  | Same as 6.6.3G.1 |  | Same as 6.6.3G.1 |  | Formula: Minimum Requirement + TT |  |
| 6.6.3G.2_1 Spurious emission band UE co-existence for V2X Communication / Sidelink simultaneous with E-UTRA uplink transmissions |  | Same as 6.6.3G.1 |  | Same as 6.6.3G.1 |  | Formula: Minimum Requirement + TT |  |
| 6.6.3G.3 Spurious emission for V2X Communication / Intra-band contiguous MCC operation |  | Same as 6.6.3G.1 |  | Same as 6.6.3G.1 |  |  |  |
| 6.6.3G.3_1 Spurious emission for V2X Communication / Intra-band contiguous MCC operation |  | Same as 6.6.3G.1 |  | Same as 6.6.3G.1 |  | Formula: Minimum Requirement + TT |  |
| 6.6.3.2 Spurious emission band UE co-existence |  | -35 dBm / 6.25kHz -36 dBm / 100kHz -41 dBm / 300kHz -37 dBm / 1MHz -40 dBm / 1MHz -50 dBm / 1MHz Frequencies as detailed in core requirement |  | 0 dB |  | Formula: Minimum Requirement + TT |  |
| 6.6.3.2A.1 Spurious emission band UE co-existence for CA (intra-band contiguous DL CA and UL CA) |  | -35 dBm / 6.25kHz -36 dBm / 100kHz -41 dBm / 300kHz -37 dBm / 1MHz -40 dBm / 1MHz -50 dBm / 1MHz -15.5 dBm / 5MHz +1.6 dBm /5MHz Frequencies as detailed in core requirement |  | 0 dB |  | Formula: Minimum Requirement + TT |  |
| 6.6.3.2A.3 Spurious emission band UE co-existence for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.6.3.2 |  | Same as 6.6.3.2 |  | Formula: Minimum Requirement + TT |  |
| 6.6.3.2A.4 Spurious emission band UE co-existence for CA (3UL CA) |  | -35 dBm / 6.25kHz -36 dBm / 100kHz -41 dBm / 300kHz -37 dBm / 1MHz -40 dBm / 1MHz -50 dBm / 1MHz -15.5 dBm / 5MHz +1.6 dBm /5MHz Frequencies as detailed in core requirement |  | 0 dB |  | Formula: Minimum Requirement + TT |  |
| 6.6.3.2A.5 Spurious emission band UE co-existence for CA (4UL CA) |  | Same as 6.6.3.2A.1 |  | Same as 6.6.3.2A.1 |  | Formula: Minimum Requirement + TT |  |
| 6.6.3.3 Additional spurious emissions |  | NS_05 1884.5MHz f 1915.7MHz: -41dBm / 300kHz NS_07 769MHz ≤ f ≤ 775MHz -57dBm / 6.25kHz NS_08 860MHz ≤ f ≤ 895MHz -40dBm / 1MHz NS_09 1475.9MHz ≤ f ≤ 1510.9MHz -35dBm / 1MHz NS_44 2620MHz ≤ f ≤ 2645MHz -15.5dBm / 5MHz 2645MHz ≤ f ≤ 2690MHz -40dBm / 1MHz |  | 0 dB 1.5dB 0 dB 0 dB 0 dB |  | Formula: Minimum Requirement + TT -41dBm / 300kHz -55.5 dBm / 6.25kHz -40dBm / 1MHz -35dBm / 1MHz -15.5dBm / 5MHz -40dBm / 1MHz |  |
| 6.6.3.3_1 Additional spurious emissions for UL 64QAM |  | Same as 6.6.3.3 |  | Same as 6.6.3.3 |  | Same as 6.6.3.3 |  |
| 6.6.3.3_2 Additional spurious emissions for UL 256QAM |  | Same as 6.6.3.3 |  | Same as 6.6.3.3 |  | Same as 6.6.3.3 |  |
| 6.6.3.3A.1 Additional spurious emissions for CA (intra-band contiguous DL CA and UL CA) |  | CA_NS_01 E-UTRA band 34: -50dBm / 1MHz 1884.5MHz f 1915.7MHz: -41dBm / 300kHz CA_NS_02 E-UTRA band 33: -50dBm / 1MHz E-UTRA band 34: -50dBm / 1MHz CA_NS_03 E-UTRA band 34: -50dBm / 1MHz E-UTRA band 39: -50dBm / 1MHz |  | 0 dB 0 dB 0 dB 0 dB 0 dB 0 dB |  | Formula: Minimum Requirement + TT -50 MHz / 1MHz -41 dBm / 300kHz -50 MHz / 1MHz -50 MHz / 1MHz -50 MHz / 1MHz -50 MHz / 1MHz |  |
| 6.6.3.3A.1_1 Additional spurious emissions for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM |  | Same as 6.6.3.3A.1 |  | Same as 6.6.3.3A.1 |  | Same as 6.6.3.3A.1 |  |
| 6.6.3.3A.1_2 Additional spurious emissions for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM |  | Same as 6.6.3.3A.1 |  | Same as 6.6.3.3A.1 |  | Same as 6.6.3.3A.1 |  |
| 6.6.3.3A.2 Additional spurious emissions for CA (inter-band DL CA and UL CA) |  | Same as 6.6.3.3 |  | Same as 6.6.3.3 |  | Same as 6.6.3.3 |  |
| 6.6.3.3A.2_1 Additional spurious emissions for CA (inter-band DL CA and UL CA) for UL 64QAM |  | Same as 6.6.3.3 |  | Same as 6.6.3.3 |  | Same as 6.6.3.3 |  |
| 6.6.3.3A.2_2 Additional spurious emissions for CA (inter-band DL CA and UL CA) for UL 256QAM |  | Same as 6.6.3.3 |  | Same as 6.6.3.3 |  | Same as 6.6.3.3 |  |
| 6.6.3.3A.4 Additional spurious emissions for CA (3UL CA) |  | Same as 6.6.3.3A.1 |  | Same as 6.6.3.3A.1 |  | Same as 6.6.3.3A.1 |  |
| 6.6.3.3A.5 Additional spurious emissions for CA (3UL CA) |  | Same as 6.6.3.3A.1 |  | Same as 6.6.3.3A.1 |  | Same as 6.6.3.3A.1 |  |
| 6.6.3B.2 Spurious emission band UE co-existence for UL-MIMO |  | Same as 6.6.3.2 |  | Same as 6.6.3.2 |  | Formula: Minimum Requirement + TT |  |
| 6.6.3B.3 Additional spurious emissions for UL-MIMO |  | Same as 6.6.3.3 |  | Same as 6.6.3.3 |  | Formula: Minimum Requirement + TT |  |
| 6.6.3F.2 Spurious emission band UE co-existence for category NB1 UE |  | Same as 6.6.3.2 |  | Same as 6.6.3.2 |  | Formula: Minimum Requirement + TT |  |
| 6.7 Transmit intermodulation |  | Intermodulation Product 5MHz -29 dBc 10MHz -35 dBc CW Interferer level = -40 dBc |  | 0 dB |  | Formula: CW interferer Minimum Requirement– TT Intermod Products limits remain unchanged. CW interferer level = -40 dBc |  |
| 6.7A.1 Transmit intermodulation for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.7 |  | Same as 6.7 |  | Same as 6.7 |  |
| 6.7A.2 Transmit intermodulation for CA (inter-band DL CA and UL CA) |  | Same as 6.7 |  | Same as 6.7 |  | Same as 6.7 |  |
| 6.7A.5 Transmit intermodulation for CA (4UL CA) |  | Same as 6.7 |  | Same as 6.7 |  | Same as 6.7 |  |
| 6.7B Transmit intermodulation for UL-MIMO |  | Same as 6.7 |  | Same as 6.7 |  | Same as 6.7 |  |
| 6.7E Transmit intermodulation for UE category 0 |  | Same as 6.7 |  | Same as 6.7 |  | Same as 6.7 |  |
| 6.7EA Transmit intermodulation for UE category M1 |  | Same as 6.7 |  | Same as 6.7 |  | Same as 6.7 |  |
| 6.7EB Transmit intermodulation for UE category 1bis |  | Same as 6.7 |  | Same as 6.7 |  | Same as 6.7 |  |
| 6.7EC Transmit intermodulation for UE category M2 |  | Same as 6.7 |  | Same as 6.7 |  | Same as 6.7 |  |
| 6.7F Transmit intermodulation for UE category NB1 and NB2 |  | Same as 6.7 |  | Same as 6.7 |  | Same as 6.7 |  |
| 6.7G.1 Transmit intermodulation for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Same as 6.7 |  | Same as 6.7 |  | Same as 6.7 |  |
| 6.7G.2 Transmit intermodulation for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Same as 6.7 |  | Same as 6.7 |  | Same as 6.7 |  |
| 6.7G.3 Transmit intermodulation for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 6.7 |  | Same as 6.7 |  | Same as 6.7 |  |
| 6.8B Time alignment error for UL-MIMO |  | The Time Alignment Error (TAE) shall not exceed 130 ns |  | 25 ns |  | Formula: Minimum Requirement+ TT |  |

## F.3.3	Measurement of receiver

Table F.3.3-1: Derivation of Test Requirements (Receiver tests)

| Test |  | Minimum Requirement in TS 36.101 |  | Test Tolerance (TT) |  | Test Requirement in TS 36.521-1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7.3 Reference sensitivity power level |  | Reference sensitivity power level: For 1.4MHz -102.2dBm -103.2dBm -105.2dBm -106.2dBm For 3MHz -99.2dBm -100.2dBm -102.2dBm For 5MHz -97dBm -98dBm -98.5dBm -99dBm -100dBm -96.5dBm Band 9 with Multi band For 10MHz -94dBm -95dBm -95.5dBm -96dBm -97dBm -93.5dBm Band 9 with Multi band For 15MHz -92.2dBm -93.2dBm -93.7dBm -94.2dBm -95.2dBm -91.7dBm Band 9 with Multi band For 20MHz -91dBm -92dBm -93dBm -94dBm -90.5dBm Band 9 with Multi band T-put limit = 95% of maximum for the Ref Meas channel |  | All cases: f ≤ 3.0GHz 0.7dB 3.0GHz < f ≤ 4.2GHz 1.0 dB |  | Formula: Reference sensitivity power level + TT T-put limit unchanged |  |
| 7.3_1 Reference sensitivity level with 4 Rx antenna ports |  | Same as 7.3 |  | Same as 7.3 |  | Same as 7.3 |  |
| 7.3A.1 Reference sensitivity level for CA (intra-band contiguous DL CA and UL CA) |  | PCC and SCC Value in Table 7.3.1-1 PCC and SCC values are each increased by ΔRIB,c in Table 7.3.3-1A or Table 7.3.1-1B where applicable T-put limit = 95% of maximum for the Ref Meas channel |  | Same as 7.3 |  | PCC and SCC Value in TS 36.101 Table 7.3.1-1+TT PCC and SCC values are each increased by ΔRIB,c in Table 7.3.3-1A where applicable T-put limit unchanged |  |
| 7.3A.2 Reference sensitivity level for CA (intra-band contiguous DL CA without UL CA) |  | Same as 7.3A.1 |  | Same as 7.3 |  | Same as 7.3A.1 |  |
| 7.3A.3 Reference sensitivity level for CA (inter-band DL CA without UL CA) |  | PCC and SCC Value in Table 7.3.1-1 Exceptions in Tables 7.3.1A-0a, 7.3.1A-0bA, 7.3.1A-0d, 7.3.1A-0eA PCC and SCC values are each increased by ΔRIB,c in Table 7.3.3-1A or Table 7.3.1-1B where applicable T-put limit = 95% of maximum for the Ref Meas channel |  | All cases: f ≤ 3.0GHz 0.7dB 3.0GHz < f ≤ 4.2GHz 1.0 dB 4.2GHz < f ≤ 6.0GHz 1.5 dB |  | PCC and SCC Value in TS 36.101 Table 7.3.1-1+TT Exceptions in TS 36.101 Tables 7.3.1A-0a, 7.3.1A-0bA, 7.3.1A-0d, 7.3.1A-0eA +TT PCC and SCC values are each increased by ΔRIB,c in Table 7.3.3-1A where applicable T-put limit unchanged |  |
| 7.3A.4 Reference sensitivity level for CA (intra-band non-contiguous DL CA without UL CA) |  | PCC Value in Table 7.3.1-1 SCC Value in Table 7.3.1-1 +ΔRIBNC in Table 7.3.1A-3 PCC and SCC values are each increased by ΔRIB,c in Table 7.3.3-1A or Table 7.3.1-1B where applicable T-put limit = 95% of maximum for the Ref Meas channel |  | Same as 7.3 |  | PCC Value in TS 36.101 Table 7.3.1-1+TT SCC Value in TS 36.101 Table 7.3.1-1+TT +ΔRIBNC in TS 36.101 Table 7.3.1A-3 PCC and SCC values are each increased by ΔRIB,c in Table 7.3.3-1A where applicable T-put limit unchanged |  |
| 7.3A.5 Reference sensitivity level for 3DL CA |  | PCC and SCCs Value in Table 7.3.1-1 Exceptions in Tables 7.3.1A-0a, 7.3.1A-0bC, 7.3.1A-0d PCC and SCCs values are each increased by ΔRIB,c in Table 7.3.1A-3, 7.3.1-1B or/and ΔRIBNC in Table 7.3.1A-3 where applicable T-put limit = 95% of maximum for the Ref Meas channel |  | Same as 7.3A.3 |  | PCC and SCC Value in TS 36.101 Table 7.3.1-1+TT Exceptions in TS 36.101 Tables 7.3.1A-0a, 7.3.1A-0bC, 7.3.1A-0d +TT PCC and SCCs values are each increased by ΔRIB,c in Table 7.3.3-1A, 7.3.3-1B or/and ΔRIBNC in TS 36.101 Table 7.3.1A-3 where applicable T-put limit unchanged |  |
| 7.3A.6 Reference sensitivity level for CA (inter-band DL CA and UL CA) |  | TBD |  | TBD |  | TBD |  |
| 7.3A.7 Reference sensitivity level for CA (intra-band non-contiguous DL CA and UL CA) |  | PCC Value in Table 7.3.1-1+ ΔR2UL_PCC in Table 7.3.1A-4 SCC Value in Table 7.3.1-1 + ΔR2UL_PCC in Table 7.3.1A-4 PCC and SCC values are each increased by ΔRIB,c in Table 7.3.3-1A or Table 7.3.1-1B where applicable T-put limit = 95% of maximum for the Ref Meas channel |  | Same as 7.3 |  | PCC Value in TS 36.101 Table 7.3.1-1+TT+ ΔR2UL_PCC in Table 7.3.1A-4 SCC Value in TS 36.101 Table 7.3.1-1+TT + ΔR2UL_SCC in Table 7.3.1A-4 PCC and SCC values are each increased by ΔRIB,c in Table 7.3.3-1A where applicable T-put limit unchanged |  |
| 7.3A.9 Reference sensitivity level for 4DL CA |  | PCC and SCCs Value in Table 7.3.1-1 Exceptions in Tables 7.3.1A-0a, 7.3.1A-0bC, 7.3.1A-0d PCC and SCCs values are each increased by ΔRIB,c in Table 7.3.1A-3, 7.3.1-1B or/and ΔRIBNC in Table 7.3.1A-3 where applicable T-put limit = 95% of maximum for the Ref Meas channel |  | Same as 7.3A.3 |  | PCC and SCC Value in TS 36.101 Table 7.3.1-1+TT Exceptions in TS 36.101 Tables 7.3.1A-0a, 7.3.1A-0bC, 7.3.1A-0d +TT PCC and SCCs values are each increased by ΔRIB,c in Table 7.3.3-1A, 7.3.3-1B or/and ΔRIBNC in TS 36.101 Table 7.3.1A-3 where applicable T-put limit unchanged |  |
| 7.3A.10 Reference sensitivity level for 5DL CA |  | PCC and SCCs Value in Table 7.3.1-1 Exceptions in Tables 7.3.1A-0a, 7.3.1A-0bC, 7.3.1A-0d PCC and SCCs values are each increased by ΔRIB,c in Table 7.3.1A-3, 7.3.1-1B or/and ΔRIBNC in Table 7.3.1A-3 where applicable T-put limit = 95% of maximum for the Ref Meas channel |  | Same as 7.3A.3 |  | PCC and SCC Value in TS 36.101 Table 7.3.1-1+TT Exceptions in TS 36.101 Tables 7.3.1A-0a, 7.3.1A-0bC, 7.3.1A-0d +TT PCC and SCCs values are each increased by ΔRIB,c in Table 7.3.3-1A, 7.3.3-1B or/and ΔRIBNC in TS 36.101 Table 7.3.1A-3 where applicable T-put limit unchanged |  |
| 7.3A.11 Reference sensitivity level for 6DL CA |  | PCC and SCCs Value in Table 7.3.1-1 Exceptions in Tables 7.3.1A-0a, 7.3.1A-0bC, 7.3.1A-0d PCC and SCCs values are each increased by ΔRIB,c in Table 7.3.1A-3, 7.3.1-1B or/and ΔRIBNC in Table 7.3.1A-3 where applicable T-put limit = 95% of maximum for the Ref Meas channel |  | Same as 7.3A.3 |  | PCC and SCC Value in TS 36.101 Table 7.3.1-1+TT Exceptions in TS 36.101 Tables 7.3.1A-0a, 7.3.1A-0bC, 7.3.1A-0d +TT PCC and SCCs values are each increased by ΔRIB,c in Table 7.3.3-1A, 7.3.3-1B or/and ΔRIBNC in TS 36.101 Table 7.3.1A-3 where applicable T-put limit unchanged |  |
| 7.3B Reference Sensitivity Level for UL-MIMO |  | Same as 7.3 |  | Same as 7.3 |  | Same as 7.3 |  |
| 7.3D.1 Reference sensitivity level for ProSe Direct Discovery |  | Values as defined in table 7.3D.0.1 |  | All cases: f ≤ 3.0GHz 0.7dB |  | Formula: Reference sensitivity power level + TT T-put limit unchanged |  |
| 7.3D.2 Reference sensitivity level for ProSe Direct Communication |  | Values as defined in table 7.3D.0.2 |  | Same as 7.3D.1 |  | Same as 7.3D.1 |  |
| 7.3E Reference Sensitivity Level for UE category 0 |  | Same as 7.3 |  | Same as 7.3 |  | Same as 7.3 |  |
| 7.3EA Reference Sensitivity Level for UE category M1 |  | Reference sensitivity power level as specified T-put limit = 95% of maximum for the Ref Meas channel |  | All cases: f ≤ 3.0GHz 0.7dB 3.0GHz < f ≤ 4.2GHz 1.0 dB |  | Formula: Reference sensitivity power level + TT T-put limit unchanged |  |
| 7.3EB Reference sensitivity level for UE category 1bis |  | Same as 7.3 |  | Same as 7.3 |  | Same as 7.3 |  |
| 7.3EC Reference Sensitivity Level for UE category M2 |  | Same as 7.3EA |  | Same as 7.3EA |  | Same as 7.3EA |  |
| 7.3F.1 Reference sensitivity level without repetitions for UE category NB1 and NB2 |  | Reference sensitivity power level: -108.2dBm T-put limit = 95% of maximum for the Ref Meas channel |  | f ≤ 3.0GHz 0.7dB |  | Formula: Reference sensitivity power level + TT T-put limit unchanged |  |
| 7.3F.2 Void |  |  |  |  |  |  |  |
| 7.3G.1 Reference sensitivity level for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Reference sensitivity power level: For 10MHz -90.4dBm For 20MHz -87.5dBm T-put limit = 95% of maximum for the Ref Meas channel |  | All cases: 4.2GHz < f ≤ 6GHz 1.5 dB |  | Formula: Reference sensitivity power level + TT T-put limit unchanged |  |
| 7.3G.2 Reference sensitivity level for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Reference sensitivity power level for V2X sidelink: For 10MHz -90.4dBm For 20MHz -87.5dBm Reference sensitivity power level for E-UTRA: For 1.4MHz -101.7dBm -102.2dBm For 3MHz -98.7dBm -99.2dBm -100.2dBm For 5MHz -97dBm -97.2dBm -98dBm -98.5dBm -100dBm For 10MHz -90.4dBm -94dBm -94.2dBm -95dBm -95.5dBm -97dBm For 15MHz -91.2dBm -92.2dBm -93.2dBm -93.7dBm -95.2dBm For 20MHz -87.5dBm -90dBm -91dBm -92dBm -94dBm T-put limit = 95% of maximum for the Ref Meas channel |  | All cases: 4.2GHz < f ≤ 6GHz 1.5 dB f ≤ 3.0GHz 0.7dB 3.0GHz < f ≤ 4.2GHz 1.0 dB |  | Formula: Reference sensitivity power level + TT T-put limit unchanged |  |
| 7.3G.3 Reference sensitivity level for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 7.3G.1 |  | Same as 7.3G.1 |  | Same as 7.3G.1 |  |
| 7.4 Maximum input level |  | Signal level -25dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | f ≤ 3.0GHz0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB |  | Formula: Maximum input level - TT Signal level: f ≤ 3.0GHz: -25.7 dBm 3.0GHz < f ≤ 4.2GHz: -26.0 dBm T-put limit unchanged Uplink power measurement window comprises four quantities: 1. UE power step size 1dB 2. UE Power step tolerance ±1dB 3. Test system power measurement at top of window: f ≤ 3.0GHz ±0.7 dB 3.0GHz < f ≤ 4.2GHz ±1.0 dB. 4. Test system power measurement at bottom of window: f ≤ 3.0GHz ±0.7 dB 3.0GHz < f ≤ 4.2GHz ±1.0 dB. Items 1 to 4 are added arithmetically: Overall UL power window size: f ≤ 3.0GHz: (1dB+1dB+0.7dB+0.7dB) =3.4dB 3.0GHz < f ≤ 4.2GHz: (1dB+1dB+1dB+1dB) =4dB Top of window is aligned to UL power requirement, hence +0dB, -3.4dB or +0dB, -4.0dB according to frequency |  |
| 7.4_1 Maximum input level with 4 Rx antenna ports |  | Same as 7.4 |  | Same as 7.4 |  | Same as 7.4 |  |
| 7.4A.1 Maximum input level for CA (intra band contiguous DL CA and UL CA) |  | Signal level for each CC as specified T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | f ≤ 3.0GHz 0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB |  | Formula: Signal level for each CC - TT T-put limit unchanged Uplink power measurement window is calculated in the same way as 7.4, but the target power for each CC is scaled according to the allocation: For PCC: 10log(P_LCRB/NRB_alloc) For SCC: 10log(S_LCRB/NRB_alloc) |  |
| 7.4A.1 H Maximum input level for SCE-L1 for CA (intra-band contiguous DL CA and UL CA) |  | Signal level for each CC as specified T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | f ≤ 3.0GHz 0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB |  | Formula: Signal level for each CC – TT Signal level: f ≤ 3.0GHz: -27.7 dBm 3.0GHz < f ≤ 4.2GHz: -28.0 dBm T-put limit unchanged Uplink power measurement window is calculated in the same way as 7.4, but the target power for each CC is scaled according to the allocation: For PCC: 10log(P_LCRB/NRB_alloc) For SCC: 10log(S_LCRB/NRB_alloc) |  |
| 7.4A.2 Maximum input level for CA (intra band contiguous DL CA without UL CA) |  | Signal level for each CC as specified T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | f ≤ 3.0GHz 0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB |  | Formula: Signal level for each CC - TT T-put limit unchanged Uplink power measurement window same as 7.4 |  |
| 7.4A.2 H Maximum input level for SCE-L1 for CA (intra-band contiguous DL CA without UL CA) |  | Signal level for each CC as specified T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | f ≤ 3.0GHz 0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB |  | Formula: Signal level for each CC - TT T-put limit unchanged Uplink power measurement window same as 7.4 |  |
| 7.4A.3 Maximum input level for CA (inter-band DL CA without UL CA) |  | Signal level -25dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | f ≤ 3.0GHz 0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB 4.2GHz < f ≤ 6.0GHz 1.5 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB |  | Formula: Maximum input level - TT Signal level: f ≤ 3.0GHz: -25.7 dBm 3.0GHz < f ≤ 4.2GHz: -26.0 dBm4.2GHz < f ≤ 6.0GHz: -26.5 dBm T-put limit unchanged Uplink power measurement window same as 7.4 |  |
| 7.4A.4 Maximum input level for CA (intra band non-contiguous DL CA without UL CA) |  | Signal level -25dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | f ≤ 3.0GHz 0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB |  | Formula: Maximum input level - TT Signal level: f ≤ 3.0GHz: -25.7 dBm 3.0GHz < f ≤ 4.2GHz: -26.0 dBm T-put limit unchanged Uplink power measurement window same as 7.4 |  |
| 7.4A.5 Maximum input level for 3DL CA |  | Signal level for each CC as specified T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | f ≤ 3.0GHz 0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB 4.2GHz < f ≤ 6.0GHz 1.5 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB |  | Formula: Signal level for each CC - TT T-put limit unchanged Uplink power measurement window same as 7.4 |  |
| 7.4A.5_H Maximum input level for SCE-L1 for 3DL CA |  | Signal level for each CC as specified T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | f ≤ 3.0GHz 0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB |  | Formula: Signal level for each CC - TT T-put limit unchanged Uplink power measurement window is calculated in the same way as 7.4 |  |
| 7.4A.7 Maximum input level for for 4DL CA |  | Signal level for each CC as specified T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | f ≤ 3.0GHz 0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB 4.2GHz < f ≤ 6.0GHz 1.5 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB |  | Formula: Signal level for each CC - TT T-put limit unchanged Uplink power measurement window is calculated in the same way as 7.4 |  |
| 7.4A.7_H Maximum input level for 4DL CA for 256QAM in DL |  | Same as 7.4A.7 |  | Same as 7.4A.7 |  | Same as 7.4A.7 |  |
| 7.4A.8 Maximum input level for for 5DL CA |  | Signal level for each CC as specified T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | f ≤ 3.0GHz 0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB 4.2GHz < f ≤ 6.0GHz 1.5 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB |  | Formula: Signal level for each CC - TT T-put limit unchanged Uplink power measurement window is calculated in the same way as 7.4 |  |
| 7.4A.8_H Maximum input level for 5DL CA for 256QAM in DL |  | Signal level for each CC as specified T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | f ≤ 3.0GHz 0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB |  | Formula: Signal level for each CC - TT T-put limit unchanged Uplink power measurement window is calculated in the same way as 7.4 |  |
| 7.4A.9 Maximum input level for for 6DL CA |  | Signal level for each CC as specified T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | f ≤ 3.0GHz 0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB 4.2GHz < f ≤ 6.0GHz 1.5 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB |  | Formula: Signal level for each CC - TT T-put limit unchanged Uplink power measurement window is calculated in the same way as 7.4 |  |
| 7.4A.9_H Maximum input level for 6DL CA for 256QAM in DL |  | Signal level for each CC as specified T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | f ≤ 3.0GHz 0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB |  | Formula: Signal level for each CC - TT T-put limit unchanged Uplink power measurement window is calculated in the same way as 7.4 |  |
| 7.4B Maximum Input Level for UL-MIMO |  | Same as 7.4 |  | Same as 7.4 |  | Same as 7.4 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 7.4D.1 Maximum input level for ProSe Direct Discovery |  | Signal level -22dBm T-put limit = 95% of maximum for the Ref Meas channel |  | f ≤ 3.0GHz 0.7 dB |  | Formula: Maximum input level - TT Signal level: f ≤ 3.0GHz: -22.7 dBm T-put limit unchanged |  |
| 7.4D.2 Maximum input level for ProSe Direct Communication |  | Same as 7.4D.1 |  | Same as 7.4D.1 |  | Same as 7.4D.1 |  |
| 7.4E Maximum input level for UE category 0 |  | Same as 7.4 |  | Same as 7.4 |  | Same as 7.4 |  |
| 7.4EA Maximum input level for UE category M1 |  | Same as 7.4 |  | Same as 7.4 |  | Same as 7.4 |  |
| 7.4EB Maximum input level for UE category 1bis |  | Same as 7.4 |  | Same as 7.4 |  | Same as 7.4 |  |
| 7.4EC Maximum input level for UE category M2 |  | Same as 7.4 |  | Same as 7.4 |  | Same as 7.4 |  |
| 7.4_H Maximum input level for SCE-L1 |  | Signal level -27dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | f ≤ 3.0GHz 0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB |  | Formula: Maximum input level - TT Signal level: f ≤ 3.0GHz: -27.7 dBm 3.0GHz < f ≤ 4.2GHz: -28.0 dBm T-put limit unchanged Uplink power measurement window same as 7.4 |  |
| 7.4F Maximum imput level for UE category NB1 and NB2 |  | Signal level -25dBm T-put limit = 95% of maximum for the Ref Meas channel |  | f ≤ 3.0GHz 0.7 dB |  | Formula: Maximum input level - TT Signal level: f ≤ 3.0GHz: -25.7dBm T-put limit unchanged |  |
| 7.4G.1 Maximum input level for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Signal level -22dBm T-put limit = 95% of maximum for the Ref Meas channel |  | 4.2GHz < f ≤ 6GHz 1.5 dB |  | Formula: Maximum input level - TT Signal level: 4.2GHz < f ≤ 6GHz : -23.5dBm T-put limit unchanged |  |
| 7.4G.2 Maximum input level for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Signal level -22dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | 4.2GHz < f ≤ 6GHz 1.5 dB for V2X sidelink f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB 4.2GHz < f ≤ 6GHz 0dB, -4.3dB |  | Same as 7.4G.1 for V2X sidelink Same as 7.4 for E-UTRA |  |
| 7.4G.3 Maximum input level for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 7.4G.1 |  | Same as 7.4G.1 |  | Same as 7.4G.1 |  |
| 7.5 Adjacent Channel Selectivity (ACS) |  | Case 1: Wanted signal power, all BWs: (REFSENS + 14 dB) Interferer signal power For 1.4 MHz, 3 MHz, 5 MHz, 10 MHz BW: (REFSENS + 45.5 dB) For 15 MHz BW: (REFSENS + 42.5 dB) For 20 MHz BW: (REFSENS + 39.5 dB) Case 2: Wanted signal power For 1.4 MHz, 3 MHz, 5 MHz, 10 MHz BW: -56.5 dBm For 15 MHz BW: -53.5 dBm For 20 MHz BW: -50.5 dBm Interferer signal power, all BWs: -25 dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | 0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB |  | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4 |  |
| 7.5_1 Adjacent Channel Selectivity (ACS) with 4 Rx antenna ports |  | Same as 7.5 |  | Same as 7.5 |  | Same as 7.5 |  |
| 7.5A.1 Adjacent Channel Selectivity (ACS) for CA (intra-band contiguous DL CA and UL CA) |  | Case 1: Wanted signal power: (REFSENS + 14 dB) Interferer signal power For CA BW Class C: (Aggregated power + 22.5 dB) Case 2: Wanted signal power: -47.5 +10log(NRB,c/NRB agg) dBm Interferer signal power For CA BW Class C: -25 dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | 0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB |  | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4A.1 |  |
| 7.5A.2 Adjacent Channel Selectivity (ACS) for CA (intra band contiguous DL CA without UL CA) |  | Same as 7.5A.1 |  | Same as 7.5A.1 |  | Same as 7.5A.1 |  |
| 7.5A.3 Adjacent Channel Selectivity (ACS) for CA (inter band DL CA without UL CA) |  | Same as 7.5 for each CC |  | Same as 7.5 |  | Same as 7.5 for each CC |  |
| 7.5A.4 Adjacent Channel Selectivity (ACS) for CA (intra-band non-contiguous DL CA without UL CA) |  | Same as 7.5 for each CC |  | Same as 7.5 |  | Same as 7.5 for each CC |  |
| 7.5A.5 Adjacent Channel Selectivity (ACS) for 3DL CA |  | Intra-band contiguous DL CA Case 1: Wanted signal power: (REFSENS + 14 dB) Interferer signal power For CA BW Class B, Class C: (Aggregated power + 22.5 dB) For CA BW Class D: (Aggregated power + 20.7 dB) Case 2: Wanted signal power: For CA BW Class B: -50.5 +10log(NRB,c/NRB agg) dBm For CA BW Class C: -47.5 +10log(NRB,c/NRB agg) dBm For CA BW Class D: -43.9 +10log(NRB,c/NRB agg) dBm Interferer signal power -25 dBm Inter-band CA Same as 7.5 for each CC T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | 0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB |  | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4A.1 |  |
| 7.5A.7 Adjacent Channel Selectivity (ACS) for 4DL CA |  | Same as 7.5A.5 for each CC |  | Same as 7.5A.5 |  | Same as 7.5A.5 for each CC |  |
| 7.5A.8 Adjacent Channel Selectivity (ACS) for 5DL CA |  | Same as 7.5A.5 for each CC except for following additional requirements Intra-band contiguous DL CA Case 1: For CA BW Class F: (Aggregated power + 18.5 dB) Case 2: Wanted signal power: For CA BW Class F: -43.5 +10log(NRB,c/NRB agg) dBm |  | Same as 7.5A.5 |  | Same as 7.5A.5 for each CC |  |
| 7.5A.9 Adjacent Channel Selectivity (ACS) for 6DL CA |  | Same as 7.5A.8 |  | Same as 7.5A.5 |  | Same as 7.5A.5 for each CC |  |
| 7.5B Adjacent Channel Selectivity (ACS) for UL-MIMO |  | Same as 7.5 |  | Same as 7.5 |  | Same as 7.5 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 7.5D.1 Adjacent channel selectivity (ACS) for ProSe Direct Discovery |  | Signal levels as defined in minimum requirements T-put limit = 95% of maximum for the Ref Meas channel |  | 0 dB |  | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.5D.2 Adjacent channel selectivity (ACS) for ProSe Direct Communication |  | Signal levels as defined in minimum requirements T-put limit = 95% of maximum for the Ref Meas channel |  | Same as 7.5D.1 |  | Same as 7.5D.1 |  |
| 7.5E Adjacent Channel Selectivity (ACS) for UE category 0 |  | Same as 7.5 |  | Same as 7.5 |  | Same as 7.5 |  |
| 7.5EA Adjacent Channel Selectivity (ACS) for UE category M1 |  | Same as 7.5 |  | Same as 7.5 |  | Same as 7.5 |  |
| 7.5EB Adjacent Channel Selectivity (ACS) for UE category 1bis |  | Same as 7.5 |  | Same as 7.5 |  | Same as 7.5 |  |
| 7.5EC Adjacent Channel Selectivity (ACS) for UE category M2 |  | Same as 7.5 |  | Same as 7.5 |  | Same as 7.5 |  |
| 7.5F Adjacent Channel Selectivity (ACS) for UE category NB1 and NB2 |  | ACS 1: Wanted signal power: (REFSENS + 14 dB) Interferer signal power For GSM interferer: (REFSENS + 42 dB) For E-UTRA interferer: (REFSENS + 47 dB) ACS 2: Wanted signal power For GSM interferer: -53 dBm For E-UTRA interferer: -58 dBm Interferer signal power: -25 dBm T-put limit = 95% of maximum for the Ref Meas channel |  | 0 dB |  | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.5G.1 Adjacent channel selectivity (ACS) for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Case 1: Wanted signal power, all BWs: (REFSENS + 14 dB) Interferer signal power For 10 MHz BW: (REFSENS + 45.5 dB) For 20 MHz BW: (REFSENS + 39.5 dB) Case 2: Wanted signal power For 10 MHz BW: -56.5 dBm For 20 MHz BW: -50.5 dBm Interferer signal power, all BWs: -25 dBm T-put limit = 95% of maximum for the Ref Meas channel |  | 0 dB |  | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.5G.2 Adjacent channel selectivity (ACS) for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Case 1: Wanted signal power, all BWs: (REFSENS + 14 dB) Interferer signal power For 10 MHz BW: (REFSENS + 45.5 dB) For 20 MHz BW: (REFSENS + 39.5 dB) Case 2: Wanted signal power For 10 MHz BW: -56.5 dBm For 20 MHz BW: -50.5 dBm Interferer signal power, all BWs: -25 dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | 0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB 4.2GHz < f ≤ 6GHz 0dB, -4.3dB |  | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.5G.3 Adjacent channel selectivity (ACS) for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 7.5G.1 |  | Same as 7.5G.1 |  | Same as 7.5G.1 |  |
| 7.6.1 In-band blocking |  | Wanted signal power: (REFSENS + BW dependent value) Interferer signal power: -56dBm or -44dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | 0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB |  | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4 |  |
| 7.6.1_1 In-band blocking with 4 Rx antenna ports |  | Same as 7.6.1 |  | Same as 7.6.1 |  | Same as 7.6.1 |  |
| 7.6.1A.1 In-band blocking for CA (intra band contiguous DL CA and UL CA) |  | Wanted signal power: (REFSENS + CA BW Class specific value) Interferer signal power: -56dBm or -44dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | Same as 7.6.1 |  | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4A.1 |  |
| 7.6.1A.2 In-band blocking for CA (intra band contiguous DL CA without UL CA) |  | Same as 7.6.1A.1 |  | Same as 7.6.1A.1 |  | Same as 7.6.1A.1 |  |
| 7.6.1A.3 In-band blocking for CA (inter band DL CA without UL CA) |  | Same as 7.6.1A.1 |  | Same as 7.6.1A.1 |  | Same as 7.6.1A.1 |  |
| 7.6.1A.4 In-band blocking for CA (intra-band non-contiguous DL CA without UL CA) |  | Same as 7.6.1A.1 |  | Same as 7.6.1A.1 |  | Same as 7.6.1A.1 |  |
| 7.6.1A.5 In-band blocking for 3DL CA |  | Same as 7.6.1A.1 |  | Same as 7.6.1A.1 |  | Same as 7.6.1A.1 |  |
| 7.6.1A.7 In-band blocking for 4DL CA |  | Same as 7.6.1A.1 |  | Same as 7.6.1A.1 |  | Same as 7.6.1A.1 |  |
| 7.6.1A.8 In-band blocking for 5DL CA |  | Same as 7.6.1A.1 |  | Same as 7.6.1A.1 |  | Same as 7.6.1A.1 |  |
| 7.6.1A.9 In-band blocking for 6DL CA |  | Same as 7.6.1A.1 |  | Same as 7.6.1A.1 |  | Same as 7.6.1A.1 |  |
| 7.6.1A.10 In-band blocking for 7DL CA |  | Same as 7.6.1A.1 |  | Same as 7.6.1A.1 |  | Same as 7.6.1A.1 |  |
| 7.6.1B In-band blocking for UL-MIMO |  | Same as 7.6.1 |  | Same as 7.6.1 |  | Same as 7.6.1 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 7.6.1D.1 In-band blocking for ProSe Direct Discovery |  | Signal levels as defined in minimum requirements T-put limit = 95% of maximum for the Ref Meas channel |  | 0 dB |  | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6.1D.2 In-band blocking for ProSe Direct Communication |  | Signal levels as defined in minimum requirements T-put limit = 95% of maximum for the Ref Meas channel |  | Same as 7.6.1D.1 |  | Same as 7.6.1D.1 |  |
| 7.6.1E In-band blocking for UE category 0 |  | Same as 7.6.1 |  | Same as 7.6.1 |  | Same as 7.6.1 |  |
| 7.6.1F In-band blocking for category NB1 and NB2 |  | Wanted signal power: (REFSENS + 6dB) Interferer signal power: -56dBm or -44dBm T-put limit = 95% of maximum for the Ref Meas channel |  | 0 dB |  | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6.1EA In-band blocking for UE category M1 |  | Wanted signal power: (REFSENS + CA BW Class specific value) Interferer signal power: -56dBm or -44dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | 0 dB f ≤ 3.0GHz 0dB, -3.4dB |  | Same as 7.6.1 |  |
| 7.6.1EB In-band blocking for UE category 1bis |  | Same as 7.6.1 |  | Same as 7.6.1 |  | Same as 7.6.1 |  |
| 7.6.1EC In-band blocking for UE category M2 |  | Same as 7.6.1EA |  | Same as 7.6.1EA |  | Same as 7.6.1 |  |
| 7.6.1G.1 In-band blocking for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Wanted signal power: (REFSENS + BW dependent value) Interferer signal power: -56dBm or -44dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | 0 dB |  | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6.1G.2 In-band blocking for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Wanted signal power: (REFSENS + BW dependent value) Interferer signal power: -56dBm or -44dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | 0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB 4.2GHz < f ≤ 6GHz 0dB, -4.3dB |  | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6.1G.3 In-band blocking for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 7.6.1G.1 |  | Same as 7.6.1G.1 |  | Same as 7.6.1G.1 |  |
| 7.6.2 Out of-band blocking |  | Wanted signal power: (REFSENS + BW dependent value) Interferer signal power: -44dBm, -30dBm or -15dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | 0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB |  | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4 |  |
| 7.6.2_1 Out of-band blocking with 4 Rx antenna ports |  | Same as 7.6.2 |  | Same as 7.6.2 |  | Same as 7.6.2 |  |
| 7.6.2A.1 Out-of-band blocking for CA (intra band contiguous DL CA and UL CA) |  | Wanted signal power: (REFSENS + CA BW Class specific value) Interferer signal power: -44dBm, -30dBm or -15dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | Same as 7.6.2 |  | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4A.1 |  |
| 7.6.2A.2 Out-of-band blocking for CA (intra band contiguous DL CA without UL CA) |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.6.2A.3 Out-of-band blocking for CA (inter band DL CA without UL CA) |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.6.2A.4 Out-of-band blocking for CA (intra-band non-contiguous DL CA without UL CA) |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.6.2A.5 Out-of-band blocking for 3DL CA |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.6.2A.7 Out-of-band blocking for 4DL CA |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.6.2A.8 Out-of-band blocking for 5DL CA |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.6.2A.9 Out-of-band blocking for 6DL CA |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.6.2A.10 Out-of-band blocking for 7DL CA |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.6.2B Out-of-band blocking for UL-MIMO |  | Same as 7.6.2 |  | Same as 7.6.2 |  | Same as 7.6.2 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 7.6.2D.1 Out-of-band blocking for ProSe Direct Discovery |  | Signal levels as defined in minimum requirements T-put limit = 95% of maximum for the Ref Meas channel |  | 0 dB |  | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6.2D.2 Out-of-band blocking for ProSe Direct Communication |  | Signal levels as defined in minimum requirements T-put limit = 95% of maximum for the Ref Meas channel |  | Same as 7.6.2D.1 |  | Same as 7.6.1D.1 |  |
| 7.6.2E Out of-band blocking for UE category 0 |  | Same as 7.6.2 |  | Same as 7.6.2 |  | Same as 7.6.2 |  |
| 7.6.2F Out of-band blocking for category NB1 and NB2 |  | Wanted signal power: (REFSENS + 6dB) Interferer signal power: -44dBm, -30dBm or -15dBm T-put limit = 95% of maximum for the Ref Meas channel |  | 0 dB |  | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6.2G.1 Out-of-band blocking for V2X Communication/ Non-concurrent with E-UTRA uplink transmissions |  | Wanted signal power: (REFSENS + BW dependent value) Interferer signal power: -44dBm, -30dBm or -15dBm T-put limit = 95% of maximum for the Ref Meas channel |  | 0 dB |  | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6.2G.2 Out-of-band blocking for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Wanted signal power: (REFSENS + BW dependent value) Interferer signal power: -44dBm, -30dBm or -15dBm T-put limit = 95% of maximum for the Ref Meas channel |  | 0 dB |  | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6.2G.3 Out-of-band blocking for V2X Communication / Intra-band contiguous multi-carrier operation |  | Wanted signal power: (REFSENS + BW dependent value) Interferer signal power: -44dBm, -30dBm or -15dBm T-put limit = 95% of maximum for the Ref Meas channel |  | 0 dB |  | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6.2EA Out-of-band blocking for UE category M1 |  | Wanted signal power: (REFSENS + BW dependent value) Interferer signal power: -44dBm, -30dBm or -15dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | 0 dB f ≤ 3.0GHz 0dB, -3.4dB |  | Same as 7.6.2 |  |
| 7.6.2EB Out-of-band blocking for UE category 1bis |  | Same as 7.6.2 |  | Same as 7.6.2 |  | Same as 7.6.2 |  |
| 7.6.2EC Out-of-band blocking for UE category M2 |  | Same as 7.6.2EA |  | Same as 7.6.2EA |  | Same as 7.6.2 |  |
| 7.6.3 Narrow band blocking |  | Wanted signal power,: (REFSENS + BW dependent value) Interferer signal power: -55dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | 0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB |  | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4 |  |
| 7.6.3_1 Narrow band blocking with 4 Rx antenna ports |  | Same as 7.6.3 |  | Same as 7.6.3 |  | Same as 7.6.3 |  |
| 7.6.3A.1 Narrow band blocking for CA (intra band contiguous DL CA and UL CA) |  | Wanted signal power: (REFSENS + CA BW Class specific value) Interferer signal power: -55dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | Same as 7.6.3 |  | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4A.1 |  |
| 7.6.3A.2 Narrow band blocking for CA (intra band contiguous DL CA without UL CA) |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.3 Narrow band blocking for CA (inter band DL CA without UL CA) |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.4 Narrow band blocking for CA (intra-band non-contiguous DL CA without UL CA) |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.5 Narrow band blocking for 3DL CA |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.7 Narrow band blocking for 4DL CA |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.8 Narrow band blocking for 5DL CA |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.9 Narrow band blocking for 6DL CA |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.10 Narrow band blocking for 7DL CA |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3B Narrow band blocking for UL-MIMO |  | Same as 7.6.3 |  | Same as 7.6.3 |  | Same as 7.6.3 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 7.6.3D.1 Narrow band blocking for ProSe Direct Discovery |  | Signal levels as defined in minimum requirements T-put limit = 95% of maximum for the Ref Meas channel |  | 0 dB |  | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6.3D.2 Narrow band blocking for ProSe Direct Communication |  | Signal levels as defined in minimum requirements T-put limit = 95% of maximum for the Ref Meas channel |  | Same as 7.6.3D.1 |  | Same as 7.6.3D.1 |  |
| 7.6.3E Narrow band blocking for UE category 0 |  | Same as 7.6.3 |  | Same as 7.6.3 |  | Same as 7.6.3 |  |
| 7.6.3EA Narrow band blocking for UE category M1 |  | Wanted signal power,: (REFSENS + BW dependent value) Interferer signal power: -55dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | 0 dB f ≤ 3.0GHz 0dB, -3.4dB |  | Same as 7.6.3 |  |
| 7.6.3EB Narrow band blocking for UE category 1bis |  | Same as 7.6.3 |  | Same as 7.6.3 |  | Same as 7.6.3 |  |
| 7.6.3EC Narrow band blocking for UE category M2 |  | Same as 7.6.3EA |  | Same as 7.6.3EA |  | Same as 7.6.3 |  |
| 7.7 Spurious response |  | Wanted signal power: (REFSENS + BW dependent value) Interferer signal power: -44dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | 0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB |  | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4 |  |
| 7.7_1 Spurious response with 4 Rx antenna ports |  | Same as 7.7 |  | Same as 7.7 |  | Same as 7.7 |  |
| 7.7A.1 Spurious response for CA (intra band contiguous DL CA and UL CA) |  | Wanted signal power: (REFSENS + CA BW Class specific value) Interferer signal power: -44dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | Same as 7.7 |  | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4A.1 |  |
| 7.7A.2 Spurious response for CA (intra band contiguous DL CA without UL CA) |  | Same as 7.7A.1 |  | Same as 7.7A.1 |  | Same as 7.7A.1 |  |
| 7.7A.3 Spurious response for CA (inter band DL CA without UL CA) |  | Same as 7.7A.1 |  | Same as 7.7A.1 |  | Same as 7.7A.1 |  |
| 7.7A.4 Spurious response for CA (intra-band non-contiguous DL CA without UL CA) |  | Same as 7.7A.1 |  | Same as 7.7A.1 |  | Same as 7.7A.1 |  |
| 7.7A.5 Spurious response for 3DL CA |  | Same as 7.7A.1 |  | Same as 7.7A.1 |  | Same as 7.7A.1 |  |
| 7.7A.7 Spurious response for 4DL CA |  | Same as 7.7A.1 |  | Same as 7.7A.1 |  | Same as 7.7A.1 |  |
| 7.7A.8 Spurious response for 5DL CA |  | Same as 7.7A.1 |  | Same as 7.7A.1 |  | Same as 7.7A.1 |  |
| 7.7A.9 Spurious response for 6DL CA |  | Same as 7.7A.1 |  | Same as 7.7A.1 |  | Same as 7.7A.1 |  |
| 7.7A.10 Spurious response for 7DL CA |  | Same as 7.7A.1 |  | Same as 7.7A.1 |  | Same as 7.7A.1 |  |
| 7.7B Spurious response for UL-MIMO |  | Same as 7.7 |  | Same as 7.7 |  | Same as 7.7 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 7.7D.1 Spurious response for ProSe Direct Discovery |  | Signal levels as defined in minimum requirements T-put limit = 95% of maximum for the Ref Meas channel |  | 0 dB |  | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.7D.2 Spurious response for ProSe Direct Communication |  | Signal levels as defined in minimum requirements T-put limit = 95% of maximum for the Ref Meas channel |  | Same as 7.7D.1 |  | Same as 7.7D.1 |  |
| 7.7E Spurious response for UE category 0 |  | Same as 7.7 |  | Same as 7.7 |  | Same as 7.7 |  |
| 7.7EA Spurious response for UE category M1 |  | Same as 7.7 |  | Same as 7.7 |  | Same as 7.7 |  |
| 7.7EB Spurious response for UE category 1bis |  | Same as 7.7 |  | Same as 7.7 |  | Same as 7.7 |  |
| 7.7EC Spurious response for UE category M2 |  | Same as 7.7 |  | Same as 7.7 |  | Same as 7.7 |  |
| 7.7F Spurious response for category NB1 and NB2 |  | Wanted signal power: (REFSENS + 6 dB) Interferer signal power: -44dBm T-put limit = 95% of maximum for the Ref Meas channel |  | 0 dB |  | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.7G.1 Spurious response for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Wanted signal power: (REFSENS + BW dependent value) Interferer signal power: -44dBm T-put limit = 95% of maximum for the Ref Meas channel |  | 0 dB |  | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.7G.2 Spurious response for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Wanted signal power: (REFSENS + BW dependent value) Interferer signal power: -44dBm T-put limit = 95% of maximum for the Ref Meas channel |  | 0 dB |  | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.7G.3 Spurious response for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 7.7G.1 |  | Same as 7.7G.1 |  | Same as 7.7G.1 |  |
| 7.8.1 Wide band intermodulation |  | Wanted signal power: For 1.4 MHz BW: (REFSENS + 12 dB) For 3 MHz BW: (REFSENS + 8 dB) For 5 MHz and 10MHz BW: (REFSENS + 6 dB) For 15 MHz BW: (REFSENS + 7 dB) For 20 MHz BW: (REFSENS + 9 dB) CW Interferer power, all BWs: -46 dBm Modulated Interferer power:, all BWs: -46 dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | 0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB |  | Formula: Wanted signal power +TT CW Interferer signal power unchanged Modulated Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4 |  |
| 7.8.1_1 Wide band Intermodulation with 4 Rx antenna ports |  | Same as 7.8.1 |  | Same as 7.8.1 |  | Same as 7.8.1 |  |
| 7.8.1A.1 Wideband intermodulation for CA (intra band contiguous DL CA and UL CA) |  | Wanted signal power: (REFSENS + CA BW Class specific value) CW Interferer power, all BWs: -46 dBm Modulated Interferer power:, all BWs: -46 dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power |  | 0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB |  | Formula: Wanted signal power +TT CW Interferer signal power unchanged Modulated Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4 A.1 |  |
| 7.8.1A.2 Wideband intermodulation for CA (intra band contiguous DL CA without UL CA) |  | Same as 7.8.1A.1 |  | Same as 7.8.1A.1 |  | Same as 7.8.1A.1 |  |
| 7.8.1A.3 Wideband intermodulation for CA (inter band DL CA without UL CA) |  | Same as 7.8.1A.1 |  | Same as 7.8.1A.1 |  | Same as 7.8.1A.1 |  |
| 7.8.1A.4 Wideband intermodulation for CA (intra band non-contiguous DL CA without UL CA) |  | Same as 7.8.1A.1 |  | Same as 7.8.1A.1 |  | Same as 7.8.1A.1 |  |
| 7.8.1A.5 Wideband intermodulation for 3DL CA |  | Same as 7.8.1A.1 |  | Same as 7.8.1A.1 |  | Same as 7.8.1A.1 |  |
| 7.8.1A.7 Wideband intermodulation for 4 DL CA |  | Same as 7.8.1A.1 |  | Same as 7.8.1A.1 |  | Same as 7.8.1A.1 |  |
| 7.8.1A.8 Wideband intermodulation for 5 DL CA |  | Same as 7.8.1A.1 |  | Same as 7.8.1A.1 |  | Same as 7.8.1A.1 |  |
| 7.8.1A.9 Wideband intermodulation for 6 DL CA |  | Same as 7.8.1A.1 |  | Same as 7.8.1A.1 |  | Same as 7.8.1A.1 |  |
| 7.8.1B Wide band intermodulation for UL-MIMO |  | Same as 7.8.1 |  | Same as 7.8.1 |  | Same as 7.8.1 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 7.8.1D.1 Wide band Intermodulation for ProSe Direct Discovery |  | Signal levels as defined in minimum requirements T-put limit = 95% of maximum for the Ref Meas channel |  | 0 dB |  | Formula: Wanted signal power + TT CW Interferer signal power unchanged Modulated Interferer signal power unchanged T-put limit unchanged |  |
| 7.8.1D.2 Wide band Intermodulation for ProSe Direct Communication |  | Signal levels as defined in minimum requirements T-put limit = 95% of maximum for the Ref Meas channel |  | Same as 7.8.1D.1 |  | Same as 7.8.1D.1 |  |
| 7.8.1E Wide band intermodulation for UE category 0 |  | Same as 7.8.1 |  | Same as 7.8.1 |  | Same as 7.8.1 |  |
| 7.8.1EA Wide band Intermodulation for UE category M1 |  | Same as 7.8.1 |  | Same as 7.8.1 |  | Same as 7.8.1 |  |
| 7.8.1EB Wide band Intermodulation for UE category 1bis |  | Same as 7.8.1 |  | Same as 7.8.1 |  | Same as 7.8.1 |  |
| 7.8.1EC Wide band Intermodulation for UE category M2 |  | Same as 7.8.1 |  | Same as 7.8.1 |  | Same as 7.8.1 |  |
| 7.8.1F Wide band intermodulation for category NB1 and NB2 |  | Wanted signal power: (REFSENS + 6 dB) CW Interferer power: -46 dBm E-UTRA Interferer power: -46 dBm T-put limit = 95% of maximum for the Ref Meas channel |  | 0 dB |  | Formula: Wanted signal power +TT CW Interferer signal power unchanged Modulated Interferer signal power unchanged T-put limit unchanged |  |
| 7.8.1G.1 Wide band Intermodulation for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Wanted signal power: For 10MHz BW: (REFSENS + 6 dB) For 20 MHz BW: (REFSENS + 9 dB) CW Interferer power, all BWs: -46 dBm Modulated Interferer power:, all BWs: -46 dBm T-put limit = 95% of maximum for the Ref Meas channel |  | 0 dB |  | Formula: Wanted signal power +TT CW Interferer signal power unchanged Modulated Interferer signal power unchanged T-put limit unchanged |  |
| 7.8.1G.2 Wide band Intermodulation for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Wanted signal power on E-UTRA channel: For 1.4 MHz BW: (REFSENS + 12 dB) For 3 MHz BW: (REFSENS + 8 dB) For 5 MHz and 10MHz BW: (REFSENS + 6 dB) For 15 MHz BW: (REFSENS + 7 dB) For 20 MHz BW: (REFSENS + 9 dB) Wanted signal power on V2X Channel: For 10MHz BW: (REFSENS + 6 dB) For 20 MHz BW: (REFSENS + 9 dB) CW Interferer power, all BWs: -46 dBm Modulated Interferer power:, all BWs: -46 dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power on E-UTRA channel |  | 0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB |  | Formula: Wanted signal power +TT CW Interferer signal power unchanged Modulated Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.8.1 |  |
| 7.8.1G.3 Wide band Intermodulation for V2X Communication / Intra-band contiguous multi-carrier operation |  | Wanted signal power: REFSENS + V2X Bandwidth Class specific value CW Interferer power, all BWs: -46 dBm Modulated Interferer power:, all BWs: -46 dBm T-put limit = 95% of maximum for the Ref Meas channel |  | 0 dB |  | Formula: Wanted signal power +TT CW Interferer signal power unchanged Modulated Interferer signal power unchanged T-put limit unchanged |  |
| 7.9 Spurious emissions |  | 30MHz f < 1GHz: -57dBm / 100kHz 1GHz f 12.75 GHz: -47dBm / 1MHz 12.75GHz f 19 GHz: -47dBm / 1MHz |  | 0 dB |  | Formula: Minimum Requirement + TT |  |
| 7.9_1 Spurious emissions with 4 Rx antenna ports |  | Same as 7.9 |  | Same as 7.9 |  | Same as 7.9 |  |
| 7.9A Spurious emissions for CA |  | 30MHz f < 1GHz: -57dBm / 100kHz 1GHz f 12.75 GHz: -47dBm / 1MHz |  | 0 dB |  | Formula: Minimum Requirement + TT |  |
| 7.9E Spurious emissions for UE category 0 |  | Same as 7.9 |  | Same as 7.9 |  | Same as 7.9 |  |
| 7.9EA Spurious emissions for UE category M1 |  | Same as 7.9 |  | Same as 7.9 |  | Same as 7.9 |  |
| 7.9EB Spurious emissions for UE category 1bis |  | Same as 7.9 |  | Same as 7.9 |  | Same as 7.9 |  |
| 7.9EC Spurious emissions for UE category M2 |  | Same as 7.9 |  | Same as 7.9 |  | Same as 7.9 |  |
| 7.9F Spurious emissions for category NB1 and NB2 |  | Same as 7.9 |  | Same as 7.9 |  | Same as 7.9 |  |
| 7.9G.1 Spurious emissions for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | 30MHz f < 1GHz: -57dBm / 100kHz 1GHz f 12.75 GHz: -47dBm / 1MHz 12.75GHz f 26 GHz: -47dBm / 1MHz |  | 0 dB |  | Formula: Minimum Requirement + TT |  |

## F.3.4	Measurement of performance requirements

Table F.3.4-1: Derivation of Test Requirements (performance tests)

| Test |  | Minimum Requirement in TS 36.101 |  | Test Tolerance (TT) |  | Test Requirement in TS 36.521-1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8.2.1.1.1 Multiple PRBs - Prop’n Condition EVA5 - Prop’n Condition ETU70 - Prop’n Condition ETU300 |  | SNRs as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.1.1 Multiple PRBs - Prop’n Condition HST |  | SNR as specified |  | 0.6dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.1.1 Single PRB - Prop’n Condition ETU70 |  | SNRs as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.1.1_1 Multiple PRBs - Prop'n Condition EVA5 - Prop'n Condition ETU70 - Prop'n Condition ETU300 |  | SNRs as specified |  | Same as 8.2.1.1.1 Multiple PRBs Propagation EVA5, ETU70, ETU300 |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.1.1_2 Multiple PRB - Prop'n Condition EVA5 |  | SNRs as specified |  | Same as 8.2.1.1.1 Multiple PRBs Propagation EVA5 |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.1.1_A.1 |  | SNRs as specified |  | Same as 8.2.1.1.1 Multiple PRBs, for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.1.1_A.2 |  | SNRs as specified |  | Same as 8.2.1.1.1 Multiple PRBs for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.1.1_A.4 |  | SNRs as specified |  | Same as 8.2.1.1.1 Multiple PRBs for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.1.1_A.5 |  | SNRs as specified |  | Same as 8.2.1.1.1 Multiple PRBs for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.1.1_A.6 |  | SNRs as specified |  | Same as 8.2.1.1.1 Multiple PRBs for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.1.1_A.7 |  | SNRs as specified |  | Same as 8.2.1.1.1 Multiple PRBs for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.1.2 Single PRB |  | SNR as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.2.1 - Prop’n Condition EVA5 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.2.1 - Prop’n Condition HST |  | SNR as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.2.1_1 - Prop'n Condition EVA5 |  | SNR as specified |  | Same as 8.2.1.2.1 |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.2.2 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.2.2_1 |  | SNR as specified |  | Same as 8.2.1.2.2 |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.2.3_C.1 |  | Noc1: -102dBm/15kHz Noc2: -98dBm/15kHz Noc3: -94.8dBm/15kHz Ês1 / Noc2: +3.4dB Ês2 / Noc2: +6dB |  | -0.1dB 0dB -0.1dB +0.9dB -0.2dB |  | Noc1: -102.1dBm/15kHz Noc2: -98dBm/15kHz Noc3: -94.9dBm/15kHz Ês1 / Noc2: +4.3dB Ês2 / Noc2: +5.8dB Formulae for Test Tolerance values: Noc1: -(Ês1/Noc1 uncertainty - Ês1/Noc2 uncertainty) Noc2: unchanged Noc3: -(Ês1/Noc3 uncertainty - Ês1/Noc2 uncertainty) Ês1 / Noc2: +Ês1/Noc2 uncertainty Ês2 / Noc2: -(Ês2/Ês1 uncertainty - Ês1/Noc2 uncertainty) T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.2.1.2.3_E.1 - Prop’n Condition EVA5 |  | Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Noc3: -93dBm/15kHz Ês1 / Noc2: +3.4dB Ês2 / Noc2: +12dB Ês3 / Noc2: +10dB |  | 0dB 0dB -0.1dB +0.9dB -0.2dB -0.2dB |  | Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Noc3: -93.1dBm/15kHz Ês1 / Noc2: +4.3dB Ês2 / Noc2: +11.8dB Ês3 / Noc2: +9.8dB The analysis is recorded in 3GPP TR 36.904 [17] T-put limit unchanged |  |
| 8.2.1.2.4 |  | Noc: -98dBm/15kHz DIP 1: -2.23dB DIP 1: -8.06dB SINR: -1.10dB |  | 0dB +0.35dB +0.85dB +0.95dB |  | Noc: -98dBm/15kHz DIP 1: -1.88dB DIP 1: -7.21dB SINR: -0.15dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.2.1.2.5 |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +15.5 dB Cell 2 Ês/Noc: +13.91 dB Cell 3 Ês/Noc: +3.34 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. |  | 0 dB +1.85 dB +0.8 dB +0.8 dB |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +17.35 dB Cell 2 Ês/Noc: +14.71 dB Cell 3 Ês/Noc: +4.14 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.2.1.2.6 |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +8.4 dB Cell 2 Ês/Noc: +3.28 dB Cell 3 Ês/Noc: +0.74 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. |  | 0 dB +1.54 dB +0.8 dB +0.8 dB |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +9.94 dB Cell 2 Ês/Noc: +4.08 dB Cell 3 Ês/Noc: +1.54 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.2.1.3.1 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.3.1_1 |  | SNR as specified |  | Same as 8.2.1.3.1 |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.3.1_A.1 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.3.1_A.2 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.3.1_A.3 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.3.1_A.4 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.3.1_A.5 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.3.1A_A.1 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.3.1B |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.3.1C |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +19.9 dB Cell 2 Ês/Noc: +12.95 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. |  | 0 dB +1.9 dB +0.8 dB |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +21.8 dB Cell 2 Ês/Noc: +13.75 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.2.1.3.2 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.3.3_C.1 |  | Noc1: -102dBm/15kHz Noc2: -98dBm/15kHz Noc3: -94.8dBm/15kHz Ês1 / Noc2: +13.3dB Ês2 / Noc2: +6dB |  | -0.1dB 0dB -0.1dB +0.9dB -0.2dB |  | Noc1: -102.1dBm/15kHz Noc2: -98dBm/15kHz Noc3: -94.9dBm/15kHz Ês1 / Noc2: +14.2dB Ês2 / Noc2: +5.8dB Formulae for Test Tolerance values: Noc1: -(Ês1/Noc1 uncertainty - Ês1/Noc2 uncertainty) Noc2: unchanged Noc3: -(Ês1/Noc3 uncertainty - Ês1/Noc2 uncertainty) Ês1 / Noc2: +Ês1/Noc2 uncertainty Ês2 / Noc2: -(Ês2/Ês1 uncertainty - Ês1/Noc2 uncertainty) T-put limit unchanged |  |
| 8.2.1.3.3_C.2 |  | Noc1: -102dBm/15kHz Noc2: -98dBm/15kHz Noc3: -94.8dBm/15kHz Ês1 / Noc2: +12.0dB Ês2 / Noc2: +6dB |  | -0.1dB 0dB -0.1dB +0.9dB -0.2dB |  | Noc1: -102.1dBm/15kHz Noc2: -98dBm/15kHz Noc3: -94.9dBm/15kHz Ês1 / Noc2: +12.9dB Ês2 / Noc2: +5.8dB Formulae for Test Tolerance values: Noc1: -(Ês1/Noc1 uncertainty - Ês1/Noc2 uncertainty) Noc2: unchanged Noc3: -(Ês1/Noc3 uncertainty - Ês1/Noc2 uncertainty) Ês1 / Noc2: +Ês1/Noc2 uncertainty Ês2 / Noc2: -(Ês2/Ês1 uncertainty - Ês1/Noc2 uncertainty) T-put limit unchanged |  |
| 8.2.1.3.3_E.1 - Prop’n Condition EVA5 |  | Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Noc3: -93dBm/15kHz Test 1 Ês1 / Noc2: + 13.9dB Ês2 / Noc2: + 9dB Ês3 / Noc2: + 7dB Test 2 Ês1 / Noc2: +22.6dB Ês2 / Noc2: +9dB +1dB Ês3 / Noc2: |  | 0dB 0dB -0.1dB +0.9dB -0.2dB -0.2dB +0.9dB -0.2dB -0.2dB |  | Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Noc3: - 93.1 dBm/15kHz Ês1 / Noc2: + 14.8 dB Ês2 / Noc2: + 8.8dB Ês3 / Noc2: + 6.8 dB Ês1 / Noc2: +23.5dB Ês2 / Noc2: +8.8dB Ês3 / Noc2: +0.8dB The analysis is recorded in 3GPP TR 36.904 [17] T-put limit unchanged |  |
| 8.2.1.4.1 |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.4.1_1 |  | SNR as specified |  | Same as 8.2.1.4.1 |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.4.1_E.1 - Prop’n Condition EPA5 |  | Noc1: -98 dBm/15kHz Noc2: -98 dBm/15kHz Noc3: -93 dBm/15kHz Ês1 / Noc2: +6.1 dB Ês2 / Noc2: +12 dB Ês3 / Noc2: +10 dB |  | 0 dB 0 dB -0.1 dB +0.9 dB -0.2 dB -0.2 dB |  | Noc1: -98 dBm/15kHz Noc2: -98 dBm/15kHz Noc3: -93.1 dBm/15kHz Ês1 / Noc2: +7.0 dB Ês2 / Noc2: +11.8 dB Ês3 / Noc2: +9.8 dB The analysis is recorded in 3GPP TR 36.904 [17] T-put limit unchanged |  |
| 8.2.1.4.1_H FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 2x2 for SCE-L1 |  | SNR as specified |  | Same as 8.2.1.4.1 |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.4.2 |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.4.2_1 |  | SNR as specified |  | Same as 8.2.1.4.2 |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.4.2_A.1 |  | SNR as specified |  | Same as 8.2.1.4.2 for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.4.2_A.2 |  | SNR as specified |  | Same as 8.2.1.4.2 for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.4.2_A.3 |  | SNR as specified |  | Same as 8.2.1.4.2 for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.4.2_A.4 |  | SNR as specified |  | Same as 8.2.1.4.2 for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.4.2_A.5 |  | SNR as specified |  | Same as 8.2.1.4.2 for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.4.2_A.6 |  | SNR as specified |  | Same as 8.2.1.4.2 for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.4.2A |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.4.3 |  | Noc: -98dBm/15kHz DIP 1: -1.73dB DIP 1: -8.66dB SINR: 0.8dB |  | 0dB +0.29dB +0.89dB +0.99dB |  | Noc: -98dBm/15kHz DIP 1: -1.44dB DIP 1: -7.77dB SINR: 1.79dB T-put limit unchanged |  |
| 8.2.1.4.3A |  | SNR as specified |  | Same as 8.2.1.4.2 for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.4.4 |  | Test 1: Noc: -98dBm/15kHz Cell 1 Ês/Noc: +17.0 dB Cell 2 Ês/Noc: +13.91 dB Cell 3 Ês/Noc: +3.34 dB Test 2: Noc: -98dBm/15kHz Cell 1 Ês/Noc: +10.1 dB Cell 2 Ês/Noc: +3.28 dB Cell 3 Ês/Noc: +0.74 dB |  | Test 1: 0 dB +1.85 dB +0.8 dB +0.8 dB Test 2: 0 dB +1.54 dB +0.8 dB +0.8 dB |  | Test 1: Noc: -98dBm/15kHz Cell 1 Ês/Noc: +18.85 dB Cell 2 Ês/Noc: +14.71 dB Cell 3 Ês/Noc: +4.14 dB T-put limit unchanged Test 2: Noc: -98dBm/15kHz Cell 1 Ês/Noc: +11.64 dB Cell 2 Ês/Noc: +4.08 dB Cell 3 Ês/Noc: +1.54 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.2.1.7.1_A.1 |  | TBD |  | TBD |  | TBD |  |
| 8.2.1.9 Multiple PRBs - Prop’n Condition HST-SFN |  | SNR as specified |  | 0.6dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.1.1 Multiple PRBs - Prop’n Condition EVA5 - Prop’n Condition ETU70 - Prop’n Condition ETU300 |  | SNRs as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.1.1 Multiple PRBs - Prop’n Condition HST |  | SNR as specified |  | 0.6dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.1.1 Single PRB - Prop’n Condition ETU70 |  | SNRs as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.1.1_1 Multiple PRBs - Prop'n Condition EVA5 - Prop'n Condition ETU70 - Prop'n Condition ETU300 |  | SNRs as specified |  | Same as 8.2.2.1.1 Multiple PRBs Propagation EVA5, ETU70, ETU300 |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.1.1_2 Multiple PRB - Prop'n Condition EVA5 |  | SNRs as specified |  | Same as 8.2.2.1.1 Multiple PRBs Propagation EVA5 |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.1.1_A.1 |  | SNRs as specified |  | 8.2.2.1.1 Multiple PRBs for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.1.1_A.2 |  | SNRs as specified |  | 8.2.2.1.1 Multiple PRBs for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.1.1_A.3 |  | SNRs as specified |  | 8.2.2.1.1 Multiple PRBs for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.1.1_A.4 |  | SNRs as specified |  | 8.2.2.1.1 Multiple PRBs for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.1.2 Single PRB |  | SNR as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.2.1 - Prop’n Condition EVA5 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.2.1 - Prop’n Condition HST |  | SNR as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.2.1_1 - Prop'n Condition EVA5 |  | SNR as specified |  | Same as 8.2.2.2.1 |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.2.2 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.2.2_1 |  | SNR as specified |  | Same as 8.2.2.2.2 |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.2.3_C.1 |  | Noc1: -102dBm/15kHz Noc2: -98dBm/15kHz Noc3: -94.8dBm/15kHz Ês1 / Noc2: +3.8dB Ês2 / Noc2: +6dB |  | -0.1dB 0dB -0.1dB +0.9dB -0.2dB |  | Noc1: -102.1dBm/15kHz Noc2: -98dBm/15kHz Noc3: -94.9dBm/15kHz Ês1 / Noc2: +4.7dB Ês2 / Noc2: +5.8dB Formulae for Test Tolerance values: Noc1: -(Ês1/Noc1 uncertainty - Ês1/Noc2 uncertainty) Noc2: unchanged Noc3: -(Ês1/Noc3 uncertainty - Ês1/Noc2 uncertainty) Ês1 / Noc2: +Ês1/Noc2 uncertainty Ês2 / Noc2: -(Ês2/Ês1 uncertainty - Ês1/Noc2 uncertainty) T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.2.2.2.3_E.1 - Prop'n Condition EVA5 |  | Noc1: [-98dBm/15kHz] Noc2: -98dBm/15kHz Noc3: -93dBm/15kHz Ês1 / Noc2: +3.5dB Ês2 / Noc2: +12dB Ês3 / Noc2: +10dB |  | 0dB 0dB -0.1dB +0.9dB -0.2dB -0.2dB |  | Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Noc3: -93.1dBm/15kHz Ês1 / Noc2: +4.4dB Ês2 / Noc2: +11.8dB Ês3 / Noc2: +9.8dB The analysis is recorded in 3GPP TR 36.904 [17] T-put limit unchanged |  |
| 8.2.2.2.4 |  | Noc: -98dBm/15kHz DIP 1: -1.73dB DIP 2: -8.66dB SINR: -1.4dB |  | 0 dB +0.29dB +0.89dB +0.99dB |  | Noc: -98dBm/15kHz DIP1: -1.44dB DIP2: -7.77 SINR: -0.41dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.2.2.2.5 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.2.6 |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +15.3 dB Cell 2 Ês/Noc: +13.91 dB Cell 3 Ês/Noc: +3.34 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. |  | 0 dB +1.85 dB +0.8 dB +0.8 dB |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +17.15 dB Cell 2 Ês/Noc: +14.71 dB Cell 3 Ês/Noc: +4.14 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.2.2.2.7 |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +8.1 dB Cell 2 Ês/Noc: +3.28 dB Cell 3 Ês/Noc: +0.74 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. |  | 0 dB +1.54 dB +0.8 dB +0.8 dB |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +9.64 dB Cell 2 Ês/Noc: +4.08 dB Cell 3 Ês/Noc: +1.54 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.2.2.3.1 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.3.1_1 |  | SNR as specified |  | Same as 8.2.2.3.1 |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.3.1_2 |  | SNR as specified |  | Same as 8.2.2.3.1 |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.3.1_A.1 |  | SNRs as specified |  | Same as 8.2.2.3.1 for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.3.1_A.2 |  | SNRs as specified |  | Same as 8.2.2.3.1 for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.3.1_A.3 |  | SNRs as specified |  | Same as 8.2.2.3.1 for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.3.1_A.4 |  | SNRs as specified |  | Same as 8.2.2.3.1 for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.3.1_A.6 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.3.1A_A.1 |  | SNRs as specified |  | Same as 8.2.2.3.1 for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.3.1B |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.3.1C |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +19.6 dB Cell 2 Ês/Noc: +12.95 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. |  | 0 dB +1.9 dB +0.8 dB |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +21.5 dB Cell 2 Ês/Noc: +13.75 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.2.2.3.2 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.3.3_C.1 |  | Noc1: -102dBm/15kHz Noc2: -98dBm/15kHz Noc3: -94.8dBm/15kHz Ês1 / Noc2: +14.0dB Ês2 / Noc2: +6dB |  | -0.1dB 0dB -0.1dB +0.9dB -0.2dB |  | Noc1: -102.1dBm/15kHz Noc2: -98dBm/15kHz Noc3: -94.9dBm/15kHz Ês1 / Noc2: +14.9dB Ês2 / Noc2: +5.8dB Formulae for Test Tolerance values: Noc1: -(Ês1/Noc1 uncertainty - Ês1/Noc2 uncertainty) Noc2: unchanged Noc3: -(Ês1/Noc3 uncertainty - Ês1/Noc2 uncertainty) Ês1 / Noc2: +Ês1/Noc2 uncertainty Ês2 / Noc2: -(Ês2/Ês1 uncertainty - Ês1/Noc2 uncertainty) T-put limit unchanged |  |
| 8.2.2.3.3_C.2 |  | Noc1: -102dBm/15kHz Noc2: -98dBm/15kHz Noc3: -94.8dBm/15kHz Ês1 / Noc2: +12.2dB Ês2 / Noc2: +6dB |  | -0.1dB 0dB -0.1dB +0.9dB -0.2dB |  | Noc1: -102.1dBm/15kHz Noc2: -98dBm/15kHz Noc3: -94.9dBm/15kHz Ês1 / Noc2: +13.1dB Ês2 / Noc2: +5.8dB Formulae for Test Tolerance values: Noc1: -(Ês1/Noc1 uncertainty - Ês1/Noc2 uncertainty) Noc2: unchanged Noc3: -(Ês1/Noc3 uncertainty - Ês1/Noc2 uncertainty) Ês1 / Noc2: +Ês1/Noc2 uncertainty Ês2 / Noc2: -(Ês2/Ês1 uncertainty - Ês1/Noc2 uncertainty) T-put limit unchanged |  |
| 8.2.2.3.3_E.1 - Prop’n Condition EVA5 |  | Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Noc3: -93dBm/15kHz Test 1 Ês1 / Noc2: + 14.2 dB Ês2 / Noc2: + 9 dB Ês3 / Noc2: + 7 dB Test 2 Ês1 / Noc2: +22.7dB Ês2 / Noc2: +9dB Ês3 / Noc2: +1dB |  | 0dB 0dB -0.1dB +0.9dB -0.2dB -0.2dB +0.9dB -0.2dB -0.2dB |  | Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Noc3: -93.1dBm/15kHz Ês1 / Noc2: + 15.1 dB Ês2 / Noc2: + 8.8 dB Ês3 / Noc2: + 6.8 dB Ês1 / Noc2: +23.6dB Ês2 / Noc2: +8.8dB Ês3 / Noc2: +0.8dB The analysis is recorded in 3GPP TR 36.904 [17] T-put limit unchanged |  |
| 8.2.2.4.1 |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.4.1_1 |  | SNR as specified |  | Same as 8.2.2.4.1 |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.4.1_E.1 - Prop'n Condition EPA5 |  | Noc1: -98 dBm/15kHz Noc2: -98 dBm/15kHz Noc3: -93 dBm/15kHz Ês1 / Noc2: +6.4 dB Ês2 / Noc2: +12 dB Ês3 / Noc2: +10 dB |  | 0 dB 0 dB -0.1 dB +0.9 dB -0.2 dB -0.2 dB |  | Noc1: -98 dBm/15kHz Noc2: -98 dBm/15kHz Noc3: -93.1 dBm/15kHz Ês1 / Noc2: +7.3 dB Ês2 / Noc2: +11.8 dB Ês3 / Noc2: +9.8 dB The analysis is recorded in 3GPP TR 36.904 [17] T-put limit unchanged |  |
| 8.2.2.4.1_H TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 2x2 for SCE-L1 |  | SNR as specified |  | Same as 8.2.2.4.1 |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.4.2 |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.4.2_1 |  | SNR as specified |  | Same as 8.2.2.4.2 |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.4.2_A.1 |  | SNR as specified |  | Same as 8.2.2.4.2 for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.4.2_A.2 |  | SNR as specified |  | Same as 8.2.2.4.2 for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.4.2_A.3 |  | SNR as specified |  | Same as 8.2.2.4.2 for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.4.2_A.4 |  | SNR as specified |  | Same as 8.2.2.4.2 for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.4.2A |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.4.3 |  | Noc: -98dBm/15kHz DIP 1: -1.73dB DIP 2: -8.66dB SINR: 1.1dB |  | 0 dB +0.29dB +0.89dB +0.99dB |  | Noc: -98dBm/15kHz DIP1: -1.44dB DIP2: -7.77 SINR: 2.09dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.2.2.4.4 |  | SNR as specified |  | Same as 8.2.2.4.2 for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.4.5 |  | Test 1: Noc: -98dBm/15kHz Cell 1 Ês/Noc: +16.1 dB Cell 2 Ês/Noc: +13.91 dB Cell 3 Ês/Noc: +3.34 dB Test 2: Noc: -98dBm/15kHz Cell 1 Ês/Noc: +9.5 dB Cell 2 Ês/Noc: +3.28 dB Cell 3 Ês/Noc: +0.74 dB |  | Test 1: 0 dB +1.85 dB +0.8 dB +0.8 dB Test 2: 0 dB +1.54 dB +0.8 dB +0.8 dB |  | Test 1: Noc: -98dBm/15kHz Cell 1 Ês/Noc: +17.95 dB Cell 2 Ês/Noc: +14.71 dB Cell 3 Ês/Noc: +4.14 dB T-put limit unchanged Test 2: Noc: -98dBm/15kHz Cell 1 Ês/Noc: +11.04 dB Cell 2 Ês/Noc: +4.08 dB Cell 3 Ês/Noc: +1.54 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.2.2.7.1_A.1 |  | TBD |  | TBD |  | TBD |  |
| 8.2.2.8.1 |  | SNRs as specified |  | Same as 8.2.2.1.1 Multiple PRBs Propagation EVA5 |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.8.2 |  | SNRs as specified |  | Same as 8.2.2.1.1 Multiple PRBs Propagation EVA5 |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2.9 Multiple PRBs - Prop’n Condition HST-SFN |  | SNR as specified |  | 0.6dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.1.1.1 |  | SNR as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.1.1.2 |  | SNR as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.1.1.3 |  | SNR as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.1.1.4 |  | SNR as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.1.1.5 |  | SNR as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.1.1.6 |  | SNR as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.1.2.1 |  | SNR as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.1.2.2 |  | SNR as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.1.2.3 |  | SNR as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.1.2.4 |  | SNR as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.1.2.5 |  | SNR as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.1.2.6 |  | SNR as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.2.1.1 |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.2.1.2 |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.2.1.3 |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.2.1.4 |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.2.1.5 |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.2.1.6 |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.2.1A |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.2.2.1 |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.2.2.2 |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.2.2.3 |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.2.2.4 |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.2.2.5 |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.2.2.6 |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.2.2A |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.3.1.1 |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.3.1.2 |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.3.1.3 |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.3.1.4 |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.3.1.5 |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.3.1.6 |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.3.2.1 |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.3.2.2 |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.3.2.3 |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.3.2.4 |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.3.2.5 |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.3.3.2.6 |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.4.1.1 |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.4.1.2 |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.3.1.1.1_D |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.3.1.1.1_H FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 without a simultaneous transmission for eDL-MIMO for SCE-L1 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.3.1.1.2_D |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.3.1.1.3 |  | Noc: -98dBm/15kHz DIP 1: -1.73dB SINR: -1.10dB |  | 0 dB +0.28dB +0.98dB |  | Noc: -98dBm/15kHz DIP1: -1.45dB SINR: -0.12dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.3.1.1.4 |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +18.5 dB Cell 2 Ês/Noc: +13.91 dB Cell 3 Ês/Noc: +3.34 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. |  | 0 dB +1.85 dB +0.8 dB +0.8 dB |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +20.35 dB Cell 2 Ês/Noc: +14.71 dB Cell 3 Ês/Noc: +4.14 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.3.1.1.5 |  | Ês1 / Noc: +14.3dB Ês2 / Noc: +13.91dB Ês3 / Noc: +3.34dB |  | +0.9dB -0.2dB 0 dB |  | Ês1 / Noc: +15.2dB Ês2 / Noc: +13.71dB Ês3 / Noc: +3.34dB Formulas for Test Tolerance values: Ês1 / Noc: +Ês1/Noc uncertainty Ês2 / Noc: -(Ês2/Ês1 uncertainty - Ês1/Noc uncertainty) T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.3.1.1.6 |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +11.5 dB Cell 2 Ês/Noc: +3.28 dB Cell 3 Ês/Noc: +0.74 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. |  | 0 dB +1.54 dB +0.8 dB +0.8 dB |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +13.04 dB Cell 2 Ês/Noc: +4.08 dB Cell 3 Ês/Noc: +1.54 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.3.1.1.7 |  | TBD |  | TBD |  | TBD |  |
| 8.3.1.1.9 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.3.1.2.1_D |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.3.1.2.1_D_1 |  | Noc: -98dBm/15kHz Ês1 / Noc: +14.2 dB Ês2 / Noc: +7.25 dB |  | 0 dB +0.8 dB -0.35 dB |  | Noc: -98dBm/15kHz Ês1 / Noc: +15.0 dB Ês2 / Noc: +6.9 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.3.1.2.2 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.3.1.3.1_F |  | SNR as specified Test 1: 2us Test 2: -0.5us |  | 0.9 dB -0.145us +0.142us |  | Formula: SNR + TT Test 1: 57Ts Test 2: -11Ts Timings: (Offset – uncertainty), then rounded to integer multiple of Ts T-put limit unchanged |  |
| 8.3.1.3.2_F |  | SNR as specified Test 1: 2us Test 2: -0.5us |  | 0.9 dB -0.145us +0.142us |  | Formula: SNR + TT Test 1: 57Ts Test 2: -11Ts Timings: (Offset – uncertainty), then rounded to integer multiple of Ts T-put limit unchanged |  |
| 8.3.1.3.3_F |  | SNR as specified Test 1: 2us Test 2: -0.5us |  | 0.9 dB -0.145us +0.142us |  | Formula: SNR + TT Test 1: 57Ts Test 2: -11Ts Timings: (Offset - uncertainty), then rounded to integer multiple of Ts T-put limit unchanged |  |
| 8.3.2.1.1 |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.3.2.1.1_1 |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.3.2.1.2 |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.3.2.1.2_D |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.3.2.1.2_H TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 without a simultaneous transmission for eDL-MIMO for SCE-L1 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.3.2.1.3 |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.3.2.1.3_D |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.3.2.1.4 |  | Noc: -98dBm/15kHz DIP 1: -1.73dB SINR: -1.0dB |  | 0 dB +0.28dB +0.98dB |  | Noc: -98dBm/15kHz DIP1: -1.45dB SINR: -0.02dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.3.2.1.5 |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +18.0 dB Cell 2 Ês/Noc: +13.91 dB Cell 3 Ês/Noc: +3.34 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. |  | 0 dB +1.85 dB +0.8 dB +0.8 dB |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +19.85 dB Cell 2 Ês/Noc: +14.71 dB Cell 3 Ês/Noc: +4.14 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.3.2.1.6 |  | Ês1 / Noc: +14.0dB Ês2 / Noc: +13.91dB Ês3 / Noc: +3.34dB |  | +0.9dB -0.2dB 0 dB |  | Ês1 / Noc: +14.9dB Ês2 / Noc: +13.71dB Ês3 / Noc: +3.34dB Formulas for Test Tolerance values: Ês1 / Noc: +Ês1/Noc uncertainty Ês2 / Noc: -(Ês2/Ês1 uncertainty - Ês1/Noc uncertainty) T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.3.2.1.7 |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +11.3 dB Cell 2 Ês/Noc: +3.28 dB Cell 3 Ês/Noc: +0.74 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. |  | 0 dB +1.54 dB +0.8 dB +0.8 dB |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +12.84 dB Cell 2 Ês/Noc: +4.08 dB Cell 3 Ês/Noc: +1.54 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.3.2.1.8 |  | TBD |  | TBD |  | TBD |  |
| 8.3.2.1.10 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.3.2.2.1 |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.3.2.2.1_D |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.3.2.2.1_D_1 |  | Noc: -98dBm/15kHz Ês1 / Noc: +14.8 dB Ês2 / Noc: +7.25 dB |  | 0 dB +0.8 dB -0.35 dB |  | Noc: -98dBm/15kHz Ês1 / Noc: +15.6 dB Ês2 / Noc: +6.9 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.3.2.2.2 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.3.2.4.1_F |  | SNR as specified Test 1: 2us Test 2: -0.5us |  | 0.9 dB -0.145us +0.142us |  | Formula: SNR + TT Test 1: 57Ts Test 2: -11Ts Timings: (Offset – uncertainty), then rounded to integer multiple of Ts T-put limit unchanged |  |
| 8.3.2.4.2_F |  | SNR as specified Test 1: 2us Test 2: -0.5us |  | 0.9 dB -0.145us +0.142us |  | Formula: SNR + TT Test 1: 57Ts Test 2: -11Ts Timings: (Offset - uncertainty), then rounded to integer multiple of Ts T-put limit unchanged |  |
| 8.3.2.4.3_F |  | SNR as specified Test 1: 2us Test 2: -0.5us |  | 0.9 dB -0.145us +0.142us |  | Formula: SNR + TT Test 1: 57Ts Test 2: -11Ts Timings: (Offset - uncertainty), then rounded to integer multiple of Ts T-put limit unchanged |  |
| 8.3.3.1.1 |  | SNR as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.3.3.1.2 |  | SNR as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.4.1.1 |  | SNR as specified |  | 0.8 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.4.1.2.1 |  | SNR as specified |  | 1.0 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.4.1.2.1_1 |  | SNR as specified |  | Same as 8.4.1.2.1 |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.4.1.2.2 |  | SNR as specified |  | 1.0 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.4.1.2.2_1 |  | SNR as specified |  | Same as 8.4.1.2.2 |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.4.1.2.3_C.1 |  | Noc1: -100.5dBm/15kHz Noc2: -98dBm/15kHz Noc3: -95.3dBm/15kHz Ês1 / Noc2: -3.9dB Ês2 / Noc2: +1.5dB |  | -0.1dB 0dB -0.1dB +0.9dB -0.2dB |  | Noc1: -100.6dBm/15kHz Noc2: -98dBm/15kHz Noc3: -95.4dBm/15kHz Ês1 / Noc2: -3dB Ês2 / Noc2: +1.3dB Formula for Test Tolerance values: Noc1: -(Ês1/Noc1 uncertainty - Ês1/Noc2 uncertainty) Noc2: unchanged Noc3: -(Ês1/Noc3 uncertainty - Ês1/Noc2 uncertainty) Ês1 / Noc2: +Ês1/Noc2 uncertainty Ês2 / Noc2: -(Ês2/Ês1 uncertainty - Ês1/Noc2 uncertainty) T-put limit unchanged |  |
| 8.4.1.2.3_C.2 |  | Noc1: -100.5dBm/15kHz Noc2: -98dBm/15kHz Noc3: -95.3dBm/15kHz Ês1 / Noc2: -4.2dB Ês2 / Noc2: +1.5dB |  | -0.1dB 0dB -0.1dB +0.9dB -0.2dB |  | Noc1: -100.6dBm/15kHz Noc2: -98dBm/15kHz Noc3: -95.4dBm/15kHz Ês1 / Noc2: -3.3dB Ês2 / Noc2: +1.3dB Formula for Test Tolerance values: Noc1: -(Ês1/Noc1 uncertainty - Ês1/Noc2 uncertainty) Noc2: unchanged Noc3: -(Ês1/Noc3 uncertainty - Ês1/Noc2 uncertainty) Ês1 / Noc2: +Ês1/Noc2 uncertainty Ês2 / Noc2: -(Ês2/Ês1 uncertainty - Ês1/Noc2 uncertainty) T-put limit unchanged |  |
| 8.4.1.2.3_E.1 - Prop'n Condition EVA5 |  | Noc1: -98 dBm/15kHz Noc2: -98 dBm/15kHz Noc3: -93 dBm/15kHz Ês1 / Noc2: -2.2 dB Ês2 / Noc2: +5 dB Ês3 / Noc2: +3 dB |  | 0 dB 0 dB -0.1 dB +0.9 dB -0.2 dB -0.2 dB |  | Noc1: -98 dBm/15kHz Noc2: -98 dBm/15kHz Noc3: -93.1 dBm/15kHz Ês1 / Noc2: -1.3 dB Ês2 / Noc2: +4.8 dB Ês3 / Noc2: +2.8 dB The analysis is recorded in 3GPP TR 36.904 [17] Pm-dsg limit unchanged |  |
| 8.4.1.2.3_E.2 - Prop'n Condition EVA5 |  | Noc1: -98 dBm/15kHz Noc2: -98 dBm/15kHz Noc3: -93 dBm/15kHz Ês1 / Noc2: -2 dB Ês2 / Noc2: +5 dB Ês3 / Noc2: +3 dB |  | 0 dB 0 dB -0.1 dB +0.9 dB -0.2 dB -0.2 dB |  | Noc1: -98 dBm/15kHz Noc2: -98 dBm/15kHz Noc3: -93.1 dBm/15kHz Ês1 / Noc2: -1.1 dB Ês2 / Noc2: +4.8 dB Ês3 / Noc2: +2.8 dB The analysis is recorded in 3GPP TR 36.904 [17] Pm-dsg limit unchanged |  |
| 8.4.2.1 |  | SNR as specified |  | 0.8 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.4.2.2.1 |  | SNR as specified |  | 1.0 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.4.2.2.1_1 |  | SNR as specified |  | Same as 8.4.2.2.1 |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.4.2.2.2 |  | SNR as specified |  | 1.0 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.4.2.2.2_1 |  | SNR as specified |  | Same as 8.4.2.2.2 |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.4.2.2.3_C.1 |  | Noc1: -100.5dBm/15kHz Noc2: -98dBm/15kHz Noc3: -95.3dBm/15kHz Ês1 / Noc2: -3.9dB Ês2 / Noc2: +1.5dB |  | -0.1dB 0dB -0.1dB +0.9dB -0.2dB |  | Noc1: -100.6dBm/15kHz Noc2: -98dBm/15kHz Noc3: -95.4dBm/15kHz Ês1 / Noc2: -3dB Ês2 / Noc2: +1.3dB Formula for Test Tolerance values: Noc1: -(Ês1/Noc1 uncertainty - Ês1/Noc2 uncertainty) Noc2: unchanged Noc3: -(Ês1/Noc3 uncertainty - Ês1/Noc2 uncertainty) Ês1 / Noc2: +Ês1/Noc2 uncertainty Ês2 / Noc2: -(Ês2/Ês1 uncertainty - Ês1/Noc2 uncertainty) T-put limit unchanged |  |
| 8.4.2.2.3_C.2 |  | Noc1: -100.5dBm/15kHz Noc2: -98dBm/15kHz Noc3: -95.3dBm/15kHz Ês1 / Noc2: -4.1dB Ês2 / Noc2: +1.5dB |  | -0.1dB 0dB -0.1dB +0.9dB -0.2dB |  | Noc1: -100.6dBm/15kHz Noc2: -98dBm/15kHz Noc3: -95.4dBm/15kHz Ês1 / Noc2: -3.2dB Ês2 / Noc2: +1.3dB Formula for Test Tolerance values: Noc1: -(Ês1/Noc1 uncertainty - Ês1/Noc2 uncertainty) Noc2: unchanged Noc3: -(Ês1/Noc3 uncertainty - Ês1/Noc2 uncertainty) Ês1 / Noc2: +Ês1/Noc2 uncertainty Ês2 / Noc2: -(Ês2/Ês1 uncertainty - Ês1/Noc2 uncertainty) T-put limit unchanged |  |
| 8.4.2.2.3_E.1 - Prop'n Condition EVA5 |  | Noc1: -98 dBm/15kHz Noc2: -98 dBm/15kHz Noc3: -93 dBm/15kHz Ês1 / Noc2: -.2 dB Ês2 / Noc2: +5 dB Ês3 / Noc2: +3 dB |  | 0 dB 0 dB -0.1 dB +0.9 dB -0.2 dB -0.2 dB |  | Noc1: -98 dBm/15kHz Noc2: -98 dBm/15kHz Noc3: -93.1 dBm/15kHz Ês1 / Noc2: -1.1 dB Ês2 / Noc2: +4.8 dB Ês3 / Noc2: +2.8 dB The analysis is recorded in 3GPP TR 36.904 [17] Pm-dsg limit unchanged |  |
| 8.4.2.2.3_E.2 - Prop'n Condition EVA5 |  | Noc1: -98 dBm/15kHz Noc2: -98 dBm/15kHz Noc3: -93 dBm/15kHz Ês1 / Noc2: -1.8 dB: Ês2 / Noc2 +5 dB Ês3 / Noc2: +3 dB |  | 0 dB 0 dB -0.1 dB +0.9 dB -0.2 dB -0.2 dB |  | Noc1: -98 dBm/15kHz Noc2: -98 dBm/15kHz Noc3: -93.1 dBm/15kHz Ês1 / Noc2: -0.9 dB Ês2 / Noc2: +4.8 dB Ês3 / Noc2: +2.8 dB The analysis is recorded in 3GPP TR 36.904 [17] Pm-dsg limit unchanged |  |
| 8.4.2.2.5 |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +16.10 dB Cell 2 Ês/Noc: +13.91 dB Cell 3 Ês/Noc: +3.34 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. |  | 0 dB +1.85 dB +0.8 dB +0.8 dB |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +17.95 dB Cell 2 Ês/Noc: +14.71 dB Cell 3 Ês/Noc: +4.14 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.4.2.2.6 |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +13.30 dB Cell 2 Ês/Noc: +13.91 dB Cell 3 Ês/Noc: +3.34 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. |  | 0 dB +1.85 dB +0.8 dB +0.8 dB |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +15.15 dB Cell 2 Ês/Noc: +14.71 dB Cell 3 Ês/Noc: +4.14 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.4.2.2.7 |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +13.70 dB Cell 2 Ês/Noc: +13.91 dB Cell 3 Ês/Noc: +3.34 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. |  | 0 dB +1.85 dB +0.8 dB +0.8 dB |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +15.55 dB Cell 2 Ês/Noc: +14.71 dB Cell 3 Ês/Noc: +4.14 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.4.2.2.8 |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +11.20 dB Cell 2 Ês/Noc: +13.91 dB Cell 3 Ês/Noc: +3.34 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. |  | 0 dB +1.85 dB +0.8 dB +0.8 dB |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +13.05 dB Cell 2 Ês/Noc: +14.71 dB Cell 3 Ês/Noc: +4.14 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.4.3.1.1 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.4.3.1.2 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.5.1.1 |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.5.1.2.1 |  | SNR as specified |  | 1.1 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.5.1.2.1_1 |  | SNR as specified |  | Same as 8.5.1.2.1 |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.5.1.2.2 |  | SNR as specified |  | 1.0 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.5.1.2.2_1 |  | SNR as specified |  | Same as 8.5.1.2.2 |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.5.1.2.3_C.1 |  | Noc1: -100.5dBm/15kHz Noc2: -98dBm/15kHz Noc3: -95.3dBm/15kHz Ês1 / Noc2: +4.6dB Ês2 / Noc2: +1.5dB |  | -0.1dB 0dB -0.1dB +0.9dB -0.2dB |  | Noc1: -100.6dBm/15kHz Noc2: -98dBm/15kHz Noc3: -95.4dBm/15kHz Ês1 / Noc2: +5.5dB Ês2 / Noc2: +1.3dB Formula for Test Tolerance values: Noc1: -(Ês1/Noc1 uncertainty - Ês1/Noc2 uncertainty) Noc2: unchanged Noc3: -(Ês1/Noc3 uncertainty - Ês1/Noc2 uncertainty) Ês1 / Noc2: +Ês1/Noc2 uncertainty Ês2 / Noc2: -(Ês2/Ês1 uncertainty - Ês1/Noc2 uncertainty) T-put limit unchanged |  |
| 8.5.1.2.3_E.1 |  | Noc1: -98 dBm/15kHz Noc2: -98 dBm/15kHz Noc3: -93 dBm/15kHz Ês1 / Noc2: +5 dB Ês2 / Noc2: +5 dB Ês3 / Noc2: +3 dB |  | 0 dB 0 dB -0.1 dB +0.9 dB -0.2 dB -0.2 dB |  | Noc1: -98 dBm/15kHz Noc2: -98 dBm/15kHz Noc3: -93.1 dBm/15kHz Ês1 / Noc2: +5.9 dB Ês2 / Noc2: +4.8 dB Ês3 / Noc2: +2.8 dB The analysis is recorded in 3GPP TR 36.904 [17] Pm-an limit unchanged |  |
| 8.5.1.2.5 |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +17.9 dB Cell 2 Ês/Noc: +13.91 dB Cell 3 Ês/Noc: +3.34 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. |  | 0 dB +1.85 dB +0.8 dB +0.8 dB |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +19.75 dB Cell 2 Ês/Noc: +14.71 dB Cell 3 Ês/Noc: +4.14 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.5.1.2.6 |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +15.80 dB Cell 2 Ês/Noc: +13.91 dB Cell 3 Ês/Noc: +3.34 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. |  | 0 dB +1.85 dB +0.8 dB +0.8 dB |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +17.65 dB Cell 2 Ês/Noc: +14.71 dB Cell 3 Ês/Noc: +4.14 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.5.1.2.7 |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +13.40 dB Cell 2 Ês/Noc: +13.91 dB Cell 3 Ês/Noc: +3.34 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. |  | 0 dB +1.85 dB +0.8 dB +0.8 dB |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +15.25 dB Cell 2 Ês/Noc: +14.71 dB Cell 3 Ês/Noc: +4.14 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.5.1.2.8 |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +15.00 dB Cell 2 Ês/Noc: +13.91 dB Cell 3 Ês/Noc: +3.34 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. |  | 0 dB +1.85 dB +0.8 dB +0.8 dB |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +16.85 dB Cell 2 Ês/Noc: +14.71 dB Cell 3 Ês/Noc: +4.14 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.5.2.1 |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.5.2.2.1 |  | SNR as specified |  | 1.1 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.5.2.2.1_1 |  | SNR as specified |  | Same as 8.5.2.2.1 |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.5.2.2.2 |  | SNR as specified |  | 1.0 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.5.2.2.2_1 |  | SNR as specified |  | Same as 8.5.2.2.2 |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.5.2.2.3_C.1 |  | Noc1: -100.5dBm/15kHz Noc2: -98dBm/15kHz Noc3: -95.3dBm/15kHz Ês1 / Noc2: +4.6dB Ês2 / Noc2: +1.5dB |  | -0.1dB 0dB -0.1dB +0.9dB -0.2dB |  | Noc1: -100.6dBm/15kHz Noc2: -98dBm/15kHz Noc3: -95.4dBm/15kHz Ês1 / Noc2: +5.5dB Ês2 / Noc2: +1.3dB Formula for Test Tolerance values: Noc1: -(Ês1/Noc1 uncertainty - Ês1/Noc2 uncertainty) Noc2: unchanged Noc3: -(Ês1/Noc3 uncertainty - Ês1/Noc2 uncertainty) Ês1 / Noc2: +Ês1/Noc2 uncertainty Ês2 / Noc2: -(Ês2/Ês1 uncertainty - Ês1/Noc2 uncertainty) T-put limit unchanged |  |
| 8.5.2.2.3_E.1 |  | Noc1: -98 dBm/15kHz Noc2: -98 dBm/15kHz Noc3: -93 dBm/15kHz Ês1 / Noc2: +5.7 dB Ês2 / Noc2: +5 dB Ês3 / Noc2: +3 dB |  | 0 dB 0 dB -0.1 dB +0.9 dB -0.2 dB -0.2 dB |  | Noc1: -98 dBm/15kHz Noc2: -98 dBm/15kHz Noc3: -93.1 dBm/15kHz Ês1 / Noc2: +6.6 dB Ês2 / Noc2: +4.8 dB Ês3 / Noc2: +2.8 dB The analysis is recorded in 3GPP TR 36.904 [17] Pm-an limit unchanged |  |
| 8.5.2.2.5 |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +16.20 dB Cell 2 Ês/Noc: +13.91 dB Cell 3 Ês/Noc: +3.34 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. |  | 0 dB +1.85 dB +0.8 dB +0.8 dB |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +18.05 dB Cell 2 Ês/Noc: +14.71 dB Cell 3 Ês/Noc: +4.14 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.5.2.2.6 |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +16.10 dB Cell 2 Ês/Noc: +13.91 dB Cell 3 Ês/Noc: +3.34 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. |  | 0 dB +1.85 dB +0.8 dB +0.8 dB |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +17.95 dB Cell 2 Ês/Noc: +14.71 dB Cell 3 Ês/Noc: +4.14 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.5.2.2.7 |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +14.00 dB Cell 2 Ês/Noc: +13.91 dB Cell 3 Ês/Noc: +3.34 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. |  | 0 dB +1.85 dB +0.8 dB +0.8 dB |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +15.85 dB Cell 2 Ês/Noc: +14.71 dB Cell 3 Ês/Noc: +4.14 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.5.2.2.8 |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +15.50 dB Cell 2 Ês/Noc: +13.91 dB Cell 3 Ês/Noc: +3.34 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. |  | 0 dB +1.85 dB +0.8 dB +0.8 dB |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +17.35 dB Cell 2 Ês/Noc: +14.71 dB Cell 3 Ês/Noc: +4.14 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.7.1.1 FDD sustained data rate performance |  | Downlink power -85dBm/15kHz |  | 0 dB |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.1.1_1 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.1.1_2 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.1.1_A.1 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.1.1_A.2 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.1.1_A.4 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.1.1_A.5 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.1.1_A.7 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.1.1_H.1 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.1.1_H.2 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 for each CC |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.1.1_H.3 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 for each CC |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.1.1_H.4 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 for each CC |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.1.1_H.5 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 for each CC |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.1.1_H.7 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 for each CC |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.2.1 TDD sustained data rate performance |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.2.1_1 |  | Same as 8.7.2.1 |  | Same as 8.7.2.1 |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.2.1_2 |  | Same as 8.7.2.1 |  | Same as 8.7.2.1 |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.2.1_A.1 |  | Same as 8.7.2.1 |  | Same as 8.7.2.1 for each CC |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.2.1_A.2 |  | Same as 8.7.2.1 |  | Same as 8.7.2.1 for each CC |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.2.1_A.3 |  | Same as 8.7.2.1 |  | Same as 8.7.2.1 for each CC |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.2.1_A.5 |  | Same as 8.7.2.1 |  | Same as 8.7.2.1 for each CC |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.2.1_A.7 |  | Same as 8.7.2.1 |  | Same as 8.7.2.1 for each CC |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.2.1_H.1 |  | Same as 8.7.2.1 |  | Same as 8.7.2.1 |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.2.1_H.2 |  | Same as 8.7.2.1 |  | Same as 8.7.2.1 for each CC |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.2.1_H.3 |  | Same as 8.7.2.1 |  | Same as 8.7.2.1 for each CC |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.2.1_H.4 |  | Same as 8.7.2.1 |  | Same as 8.7.2.1 for each CC |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.2.1_H.5 |  | Same as 8.7.2.1 |  | Same as 8.7.2.1 for each CC |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.2.1_H.7 |  | Same as 8.7.2.1 |  | Same as 8.7.2.1 for each CC |  | Formula: Downlink power + TT T-put limit unchanged |  |
| Haule13098.7.3.1 FDD sustained data rate performance for EPDCCH scheduling |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.4.1 TDD sustained data rate performance for EPDCCH scheduling |  | Same as 8.7.2.1 |  | Same as 8.7.2.1 |  | Same as 8.7.2.1 |  |
| 8.7.5.1.1 TDD FDD CA Sustained data rate performance for FDD PCell (2DL CA) |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 for each CC |  | Same as 8.7.1.1 |  |
| 8.7.5.1.2 TDD FDD CA Sustained data rate performance for FDD PCell (3DL CA) |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 for each CC |  | Same as 8.7.1.1 |  |
| 8.7.5.1.3 TDD FDD CA Sustained data rate performance for FDD PCell (4DL CA) |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 for each CC |  | Same as 8.7.1.1 |  |
| 8.7.5.1.4 TDD FDD CA Sustained data rate performance for FDD PCell (5DL CA) |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 for each CC |  | Same as 8.7.1.1 |  |
| 8.7.5.1.5 TDD FDD CA Sustained data rate performance for FDD PCell (6DL CA) |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 for each CC |  | Same as 8.7.1.1 |  |
| 8.7.5.1.6 TDD FDD CA Sustained data rate performance for FDD PCell (7DL CA) |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 for each CC |  | Same as 8.7.1.1 |  |
| 8.7.5.1_H.1 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (2DL CA) |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 for each CC |  | Same as 8.7.1.1 |  |
| 8.7.5.1_H.2 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (3DL CA) |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 for each CC |  | Same as 8.7.1.1 |  |
| 8.7.5.1_H.3 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (4DL CA) |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 for each CC |  | Same as 8.7.1.1 |  |
| 8.7.5.1_H.4 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (5DL CA) |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 for each CC |  | Same as 8.7.1.1 |  |
| 8.7.5.2.1 TDD FDD CA Sustained data rate performance for TDD PCell (2DL CA) |  | Same as 8.7.2.1 |  | Same as 8.7.2.1 for each CC |  | Same as 8.7.2.1 |  |
| 8.7.5.2.2 TDD FDD CA Sustained data rate performance for TDD PCell (3DL CA) |  | Same as 8.7.2.1 |  | Same as 8.7.2.1 for each CC |  | Same as 8.7.2.1 |  |
| 8.7.5.2.3 TDD FDD CA Sustained data rate performance for TDD PCell (4DL CA) |  | Same as 8.7.2.1 |  | Same as 8.7.2.1 for each CC |  | Same as 8.7.2.1 |  |
| 8.7.5.2.4 TDD FDD CA Sustained data rate performance for TDD PCell (5DL CA) |  | Same as 8.7.2.1 |  | Same as 8.7.2.1 for each CC |  | Same as 8.7.2.1 |  |
| 8.7.5.2.5 TDD FDD CA Sustained data rate performance for TDD PCell (6DL CA) |  | Same as 8.7.2.1 |  | Same as 8.7.2.1 for each CC |  | Same as 8.7.2.1 |  |
| 8.7.5.2.6 TDD FDD CA Sustained data rate performance for TDD PCell (7DL CA) |  | Same as 8.7.2.1 |  | Same as 8.7.2.1 for each CC |  | Same as 8.7.2.1 |  |
| 8.7.5.2_H.1 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (2DL CA) |  | Same as 8.7.2.1 |  | Same as 8.7.2.1 for each CC |  | Same as 8.7.2.1 |  |
| 8.7.5.2_H.2 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (3DL CA) |  | Same as 8.7.2.1 |  | Same as 8.7.2.1 for each CC |  | Same as 8.7.2.1 |  |
| 8.7.5.2_H.3 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (4DL CA) |  | Same as 8.7.2.1 |  | Same as 8.7.2.1 for each CC |  | Same as 8.7.2.1 |  |
| 8.7.5.2_H.4 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (5DL CA) |  | Same as 8.7.2.1 |  | Same as 8.7.2.1 for each CC |  | Same as 8.7.2.1 |  |
| 8.7.6.1 FDD sustained data rate performance for Dual Connectivity 64QAM |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.6.2 FDD sustained data rate performance for Dual Connectivity 256QAM |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.7.1 TDD sustained data rate performance for Dual Connectivity 64QAM |  | Same as 8.7.2.1 |  | Same as 8.7.2.1 |  | Same as 8.7.2.1 |  |
| 8.7.7.2 TDD sustained data rate performance for Dual Connectivity 256QAM |  | Same as 8.7.2.1 |  | Same as 8.7.2.1 |  | Same as 8.7.2.1 |  |
| 8.7.9.2 FDD sustained data rate performance for 4 layer MIMO (single carrier) |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.9.3 FDD sustained data rate performance for 4 layer MIMO (2DL CA) |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.9.4 FDD sustained data rate performance for 4 layer MIMO (3DL CA) |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.9.5 FDD sustained data rate performance for 4 layer MIMO (4DL CA) |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.9.6 FDD sustained data rate performance for 4 layer MIMO (5DL CA) |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.10.2 TDD sustained data rate performance for 4 layer MIMO (single carrier) |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.10.3 TDD sustained data rate performance for 4 layer MIMO (2DL CA) |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.10.4 TDD sustained data rate performance for 4 layer MIMO (3DL CA) |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.10.5 TDD sustained data rate performance for 4 layer MIMO (4DL CA) |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.10.6 TDD sustained data rate performance for 4 layer MIMO (5DL CA) |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.11.2 TDD FDD CA sustained data rate performance for 4 layer MIMO (2DL CA) |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.11.3 TDD FDD CA sustained data rate performance for 4 layer MIMO (3DL CA) |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.11.4 TDD FDD CA sustained data rate performance for 4 layer MIMO (4DL CA) |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.11.5 TDD FDD CA sustained data rate performance for 4 layer MIMO (5DL CA) |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.12.1.2.1 FDD as Pcell with 2DL CA (2RX) |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.12.1.2.2 FDD as Pcell with 2DL CA (4 layer MIMO) |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.12.2.2.1 LAA sustained data rate performance with TDD PCell with 2DL CA (2Rx) |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.12.2.2.2 LAA sustained data rate performance with TDD PCell for 4 layer MIMO (2DL CA) |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.8.1.1 FDD distributed EPDCCH performance |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT Pm-dsg limit unchanged |  |
| 8.8.1.2 TDD distributed EPDCCH performance |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT Pm-dsg limit unchanged |  |
| 8.8.2.1 FDD localized EPDCCH performance with TM9 |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT Pm-dsg limit unchanged |  |
| 8.8.2.2 TDD localized EPDCCH performance with TM9 |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT Pm-dsg limit unchanged |  |
| 8.8.3.1 FDD localized EPDCCH transmission with TM10 Type B quasi co-location type |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT Pm-dsg limit unchanged |  |
| 8.8.3.2 TDD localized EPDCCH transmission with TM10 Type B quasi co-location type |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT Pm-dsg limit unchanged |  |
| 8.8.4.1 |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +13.40 dB Cell 2 Ês/Noc: +13.91 dB Cell 3 Ês/Noc: +3.34 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. |  | 0 dB +1.85 dB +0.8 dB +0.8 dB |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +15.25 dB Cell 2 Ês/Noc: +14.71 dB Cell 3 Ês/Noc: +4.14 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.8.4.2 |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +14.20 dB Cell 2 Ês/Noc: +13.91 dB Cell 3 Ês/Noc: +3.34 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. |  | 0 dB +1.85 dB +0.8 dB +0.8 dB |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +16.05 dB Cell 2 Ês/Noc: +14.71 dB Cell 3 Ês/Noc: +4.14 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.8.5.1 |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +14.80 dB Cell 2 Ês/Noc: +13.91 dB Cell 3 Ês/Noc: +3.34 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. |  | 0 dB +1.85 dB +0.8 dB +0.8 dB |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +16.65 dB Cell 2 Ês/Noc: +14.71 dB Cell 3 Ês/Noc: +4.14 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.8.6.1 |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +15.90 dB Cell 2 Ês/Noc: +13.91 dB Cell 3 Ês/Noc: +3.34 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. |  | 0 dB +1.85 dB +0.8 dB +0.8 dB |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +17.75 dB Cell 2 Ês/Noc: +14.71 dB Cell 3 Ês/Noc: +4.14 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.9.1.1.1 FDD PDSCH Transmit Diversity 2x1 for UE category 0 |  | SNR as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.9.1.1.2 FDD Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols) for UE category 0 |  | SNR as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.9.1.1.3 FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category 0 |  | SNR as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.9.1.1.3_1 FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category 1bis |  | SNR as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.9.1.1.1_1 FDD PDSCH Transmit Diversity 2x1 for UE category 1bis |  | SNR as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.9.1.1.2_1 FDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x1 for UE Category 1bis |  | SNR as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.9.1.2.1 TDD PDSCH Transmit Diversity for UE category 0 |  | SNR as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.9.1.2.1_1 TDD PDSCH Transmit Diversity 2x1 for UE category 1bis |  | SNR as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.9.1.2.2 TDD Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols) for UE category 0 |  | SNR as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.9.1.2.2_1 TDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x1 for UE Category 1bis |  | SNR as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.9.1.2.3 TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category 0 |  | SNR as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.9.1.2.3_1 TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category 1bis |  | SNR as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.9.2.1.1 FDD PHICH transmit diversity for UE category 0 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.9.2.1.1_1 FDD PHICH transmit diversity for UE category 1bis |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.9.2.2.1 TDD PHICH Transmit Diversity for UE category 0 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.9.2.2.1_1 TDD PHICH transmit diversity for UE category 1bis |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.10.1.1.1 FDD PDSCH Transmit Diversity 2x4 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.10.1.1.2 FDD PDSCH Open Loop Spatial Multiplexing 2x4 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.10.1.1.3 FDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model – Enhanced Performance Requirement Type A |  | Noc: -98dBm/15kHz DIP 1: -1.73dB SINR: -2.3dB |  | 0dB +0.28dB +0.98dB |  | Noc: -98dBm/15kHz DIP 1: -1.45dB SINR: -1.32dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.10.1.1.4 FDD PDSCH Closed Loop Spatial Multiplexing 4x4 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.10.1.1.5 FDD PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 interference model – Enhanced Performance Requirement Type A |  | Noc: -98dBm/15kHz DIP 1: -1.73dB SINR: -3.0dB |  | 0 dB +0.28dB +0.98dB |  | Noc: -98dBm/15kHz DIP1: -1.45dB SINR: -2.03dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.10.1.1.6 FDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) |  | Noc: -98dBm/15kHz Ês1 / Noc: +9.2 dB Ês2 / Noc: +7.25 dB |  | 0 dB +0.8 dB -0.35 dB |  | Noc: -98dBm/15kHz Ês1 / Noc: +10.0 dB Ês2 / Noc: +6.9 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.10.1.1.7 FDD Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.10.1.1.8 FDD Closed-loop spatial multiplexing performance, 4 Layers spatial multiplexing 4 Tx antennas |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.10.1.1.9 FDD 4 Layer Spatial Multiplexing (User-Specific Reference Symbols) |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.10.1.2.1 TDD PDSCH Transmit Diversity 2x4 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.10.1.2.2 TDD PDSCH Open Loop Spatial Multiplexing 2x4 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.10.1.2.3 TDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model – Enhanced Performance Requirement Type A |  | Noc: -98dBm/15kHz DIP 1: -1.73dB SINR: -1.9dB |  | 0dB +0.28dB +0.98dB |  | Noc: -98dBm/15kHz DIP 1: -1.45dB SINR: -0.92dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.10.1.2.4 TDD PDSCH Closed Loop Spatial Multiplexing 4x4 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.10.1.2.5 FDD PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 interference model – Enhanced Performance Requirement Type A |  | Noc: -98dBm/15kHz DIP 1: -1.73dB SINR: -3.3dB |  | 0 dB +0.28dB +0.98dB |  | Noc: -98dBm/15kHz DIP1: -1.45dB SINR: -2.32dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.10.1.2.6 TDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) |  | Noc: -98dBm/15kHz Ês1 / Noc: +9.5 dB Ês2 / Noc: +7.25 dB |  | 0 dB +0.8 dB -0.35 dB |  | Noc: -98dBm/15kHz Ês1 / Noc: +10.3 dB Ês2 / Noc: +6.9 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.10.1.2.7 TDD Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.10.1.2.8 TDD Closed-loop spatial multiplexing performance, 4 Layers spatial multiplexing 4 Tx antennas |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.10.1.2.9 TDD 4 Layer Spatial Multiplexing (User-Specific Reference Symbols) |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.10.2.1.1 FDD PCFICH/PDCCH Single-antenna Port Performance 1x4 |  | SNR as specified |  | 0.8 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.10.2.1.2 FDD PCFICH/PDCCH Transmit Diversity 2x4 |  | SNR as specified |  | 1.0 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.10.2.1.3 FDD PCFICH/PDCCH Transmit Diversity 4x4 |  | SNR as specified |  | 1.0 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.10.2.2.1 TDD PCFICH/PDCCH Single-antenna Port Performance 1x4 |  | SNR as specified |  | 0.8 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.10.2.2.2 TDD PCFICH/PDCCH Transmit Diversity 2x4 |  | SNR as specified |  | 1.0 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.10.2.2.3 TDD PCFICH/PDCCH Transmit Diversity 4x4 |  | SNR as specified |  | 1.0 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.10.3.1.1 FDD PHICH Single-antenna Port Performance 1x4 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT Pm-an limit unchanged |  |
| 8.10.3.1.2 FDD PHICH Transmit Diversity 2x4 |  | SNR as specified |  | 1.1 dB |  | Formula: SNR + TT Pm-an limit unchanged |  |
| 8.10.3.1.3 FDD PHICH Transmit Diversity 4x4 |  | SNR as specified |  | 1.0 dB |  | Formula: SNR + TT Pm-an limit unchanged |  |
| 8.10.3.2.1 TDD PHICH Single-antenna Port Performance 1x4 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT Pm-an limit unchanged |  |
| 8.10.3.2.2 TDD PHICH Transmit Diversity 2x4 |  | SNR as specified |  | 1.1 dB |  | Formula: SNR + TT Pm-an limit unchanged |  |
| 8.10.3.2.3 TDD PHICH Transmit Diversity 4x4 |  | SNR as specified |  | 1.0 dB |  | Formula: SNR + TT Pm-an limit unchanged |  |
| 8.10.4.1.1 FDD distributed E2PDCCH performance 2x4 |  | SNRs as specified |  | 0.9dB |  | Formula: SNR + TT Pm-dsg limit unchanged |  |
| 8.10.4.1.2 TDD distributed EPDCCH performance 2x4 |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT Pm-dsg limit unchanged |  |
| 8.10.4.2.1 FDD localized EPDCCH performance with TM9 2x4 |  | SNRs as specified |  | 0.9dB |  | Formula: SNR + TT Pm-dsg limit unchanged |  |
| 8.10.4.2.2 TDD localized EPDCCH performance with TM9 2x4 |  | SNRs as specified |  | 0.9dB |  | Formula: SNR + TT Pm-dsg limit unchanged |  |
| 8.11.1.1.1 Closed-loop spatial multiplexing performance for UE supporting CE |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.11.1.1.2 FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE supporting CE |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.11.1.1.3.1 FDD PDSCH Transmit Diversity 2x1 for UE category M1 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.11.1.1.3.1_1 FDD PDSCH Transmit Diversity 2x1 for UE category M1 (CEModeB) |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.11.1.1.3.2 FDD and half-duplex FDD PDSCH Transmit Diversity 2x1 for UE category M2 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.11.1.1.3.2_1 FDD and half-duplex FDD PDSCH Transmit Diversity 2x1 for UE category M2 (CEModeB) |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.11.1.2.1 TDD Closed-loop spatial multiplexing performance in CE Mode A (Cell-Specific Reference Symbols) |  | Same as 8.11.1.1.1 |  | Same as 8.11.1.1.1 |  | Same as 8.11.1.1.1 |  |
| 8.11.1.2.2 TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for in CE Mode A |  | SNRs as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.11.1.2.3.1 TDD PDSCH Transmit Diversity for UE category M1 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.11.1.2.3.1_1 TDD PDSCH Transmit Diversity for UE category M1 (CEModeB) |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.11.1.2.3.2 TDD PDSCH 2 Tx Antenna Port supporting wideband transmission for UE category M2 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.11.1.2.3.2_1 TDD PDSCH 2 Tx Antenna Port supporting wideband transmission for UE category M2 (CEModeB) |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.11.2.1.1 FDD and HD-FDD demodulation of MPDCCH in CE Mode A |  | SNRs as specified |  | 0.9dB |  | Formula: SNR + TT Pm-dsg limit unchanged |  |
| 8.11.2.1.2 FDD and HD-FDD demodulation of MPDCCH in CE Mode B |  | SNRs as specified |  | 0.9dB |  | Formula: SNR + TT Pm-dsg limit unchanged |  |
| 8.11.2.2.1 TDD demodulation of MPDCCH in CE Mode A |  | SNRs as specified |  | 0.9dB |  | Formula: SNR + TT Pm-dsg limit unchanged |  |
| 8.11.2.2.2 TDD demodulation of MPDCCH in CE Mode B |  | SNRs as specified |  | 0.9dB |  | Formula: SNR + TT Pm-dsg limit unchanged |  |
| 8.12.1.1.1 Demodulation of NPDSCH (Cell-Specific Reference Symbols) in In-band mode for category NB1 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.12.1.1.2 Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone and Guard-band mode for category NB1 |  | SNRs as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.12.1.1.3 Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone for NB2 |  | SNRs as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.12.2.1.1 Demodulation of NPDCCH single-antenna performance for category NB1 |  | SNRs as specified |  | 0.8dB |  | Formula: SNR + TT Pm-dsg limit unchanged |  |
| 8.12.2.1.2 Demodulation of NPDCCH Transmit diversity performance for category NB1 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT Pm-dsg limit unchanged |  |
| 8.13.1.1.1.2 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.1.1.1.3 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.1.1.1.4 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.1.1.1.5 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.1.1.3 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.1.1.4 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.1.2.2 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.1.2.3 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.1.2.4 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.1.2.5 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.1.3.1 |  | Noc: -98dBm/15kHz DIP 1: -1.73dB SINR: -2.3dB |  | 0dB +0.28dB +0.98dB |  | Noc: -98dBm/15kHz DIP 1: -1.45dB SINR: -1.32dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.13.1.4.1 |  | Noc: -98dBm/15kHz DIP 1: -1.73dB SINR: -3.0dB |  | 0 dB +0.28dB +0.98dB |  | Noc: -98dBm/15kHz DIP1: -1.45dB SINR: -2.03dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.13.2.1.1.2 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.2.1.1.3 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.2.1.1.4 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.2.1.1.5 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.2.1.3 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.2.1.4 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.2.2.2 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.2.2.3 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.2.2.4 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.2.2.5 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.2.3.1 |  | Noc: -98dBm/15kHz DIP 1: -1.73dB SINR: -1.9dB |  | 0dB +0.28dB +0.98dB |  | Noc: -98dBm/15kHz DIP 1: -1.45dB SINR: -0.92dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.13.2.4.1 |  | Noc: -98dBm/15kHz DIP 1: -1.73dB SINR: -3.3dB |  | 0 dB +0.28dB +0.98dB |  | Noc: -98dBm/15kHz DIP1: -1.45dB SINR: -2.32dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.13.3.1.1.2 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.3.1.1.3 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.3.1.1.4 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.3.1.1.5 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.3.1.2.2 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.3.1.2.3 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.3.1.2.4 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.3.1.2.5 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.3.2.3 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.3.2.4 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.3.2.5 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.3.2.6 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.3.2.7 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.3.2.8 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.3.2.9 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.3.2.10 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.3.3.1 TDD-FDD CA PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model-Enhanced Performance Requirement Type A for FDD Pcell (2DL CA) |  | - Noc: -98dBm/15kHz - DIP 1: -1.73dB - FDD Cell 2Rx SINR: 5MHz: 0.8dB 10MHz: 1.0dB 15MHz: 0.8dB 20MHz: 1.1dB - FDD Cell 4Rx SINR: 5MHz: -3.0dB 10MHz: -2.7dB 15MHz: -2.9dB 20MHz: -2.8dB - TDD Cell 2Rx SINR: 5MHz: 0.8dB 10MHz: 1.3dB 15MHz: 1.0 dB 20MHz: 1.3dB - TDD Cell 4Rx SINR: 5MHz: -2.8dB 10MHz: -2.5dB 15MHz: -2.7dB 20MHz: -2.3dB |  | 0dB +0.28dB +0.98dB +0.98dB +0.98dB +0.98dB |  | Noc: -98dBm/15kHz DIP 1: -1.45dB - FDD Cell 2Rx SINR: 5MHz: 1.78dB 10MHz: 1.98dB 15MHz: 1.78dB 20MHz: 2.08dB - FDD Cell 4Rx SINR: 5MHz: 2.02dB 10MHz: 1.72dB 15MHz: 1.92dB 20MHz: 1.82dB - TDD Cell 2Rx SINR: 5MHz: 1.78dB 10MHz: 2.28dB 15MHz: 1.98dB 20MHz: 2.28dB - TDD Cell 4Rx SINR: 5MHz: 1.82dB 10MHz: 1.52dB 15MHz: 1.72dB 20MHz: 1.32dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.13.3.3.2 TDD-FDD CA PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model-Enhanced Performance Requirement Type A for TDD Pcell (2DL CA) |  | Same as 8.13.3.3.1 |  | Same as 8.13.3.3.1 |  | Same as 8.13.3.3.1 |  |
| 8.13.3.4.1 TDD-FDD CA PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 Interference Model-Enhanced Performance Requirement Type A for FDD Pcell (2DL CA) |  | - Noc: -98dBm/15kHz - DIP 1: -1.73dB - FDD Cell 2Rx SINR: 5MHz: -0.4dB 10MHz: 0.1dB 15MHz: -0.2dB 20MHz: -0.4dB - FDD Cell 4Rx SINR: 5MHz: -2.0dB 10MHz: -1.8dB 15MHz: -1.9dB 20MHz: -1.7dB - TDD Cell 2Rx SINR: 5MHz: -0.6dB 10MHz: -0.1dB 15MHz: -0.2dB 20MHz: -0.4dB - TDD Cell 4Rx SINR: 5MHz: -2.4dB 10MHz: -2.1dB 15MHz: -2.1dB 20MHz: -2.1dB |  | 0dB +0.28dB +0.98dB +0.98dB +0.98dB +0.98dB |  | Noc: -98dBm/15kHz DIP 1: -1.45dB - FDD Cell 2Rx SINR: 5MHz: 0.58dB 10MHz: 1.08dB 15MHz: 0.78dB 20MHz: 0.58dB - FDD Cell 4Rx SINR: 5MHz: -1.02dB 10MHz: -0.82dB 15MHz: -0.92dB 20MHz: -0.72dB - TDD Cell 2Rx SINR: 5MHz: 0.38dB 10MHz: 0.88dB 15MHz: 0.78dB 20MHz: 0.58dB - TDD Cell 4Rx SINR: 5MHz: -1.42dB 10MHz: -1.12dB 15MHz: -1.12dB 20MHz: -1.12dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 8.13.3.4.2 TDD-FDD CA PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 Interference Model-Enhanced Performance Requirement Type A for TDD Pcell (2DL CA) |  | Same as 8.13.3.4.1 |  | Same as 8.13.3.4.1 |  | Same as 8.13.3.4.1 |  |
| 8.13.3.6.1 |  | SNR as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.3.6.2 |  | SNR as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.3.7.1 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.3.7.2 |  | SNRs as specified |  | 0.9 dB for each CC |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.14.1.1.1 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.14.1.1.2 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.14.1.2.1 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.14.1.2.2 |  | SNR as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.14.2.1 |  | SNR as specified |  | 1.0 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.14.2.2 |  | SNR as specified |  | 1.0 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 10.1 |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 10.2 |  | SNR as specified |  | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 14.2 |  | SNRs as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 14.3 |  | SNRs as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 14.4 |  | SNRs as specified |  | 0.6dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 14.6 |  | SNRs as specified |  | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 14.7 |  | SNRs as specified |  | 0.6dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 14.8 |  | Downlink power -85dBm/15kHz |  | 0 dB |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 14.9 |  | SNRs as specified |  | E-UTRA: 0dB Sidelink: 0.6dB |  | E-UTRA: Same as 8.7.1.1 Sidelink: Same as 14.7 |  |
| [Other tests FFS] |  |  |  |  |  |  |  |

## F.3.5	Measurement of Channel State Information reporting

Table F.3.5-1: Derivation of Test Requirements (Channel State Information reporting tests)

| Test |  | Minimum Requirement in TS 36.101 |  | Test Tolerance (TT) |  | Test Requirement in TS 36.521-1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 9.2.1.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.1.2 TDD CQI Reporting under AWGN condit.1ions – PUCCH 1-0 |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.1.3_C.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 for eICIC (non-MBSFN ABS) |  | Values as specified Limits as in the Test Procedure |  | No test tolerances applied |  | The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.2.1.4_C.1 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 for eICIC (non-MBSFN ABS) |  | Values as specified Limits as in the Test Procedure |  | No test tolerances applied |  | The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.2.1.5_E.1 FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for feICIC (non-MBSFN ABS) |  | Values as specified Limits as in the Test Procedure |  | No test tolerances applied |  | The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.2.1.6_E.1 TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for feICIC (non-MBSFN ABS) |  | Values as specified Limits as in the Test Procedure |  | No test tolerances applied |  | The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.2.1.7 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 for 256QAM in DL |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.1.8 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 for 256QAM in DL |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.2.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-1 |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.2.2 TDD CQI Reporting under AWGN conditions – PUCCH 1-1 |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.3.1_D FDD CQI Reporting under AWGN conditions – PUCCH 1-1 for eDL-MIMO |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.3.2_D TDD CQI Reporting under AWGN conditions – PUCCH 1-1 for eDL-MIMO |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.3.1A FDD CQI Reporting under AWGN conditions – PUCCH 1-1 for FD-MIMO (With channelMeasRestriction configured) |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.3.2A TDD CQI Reporting under AWGN conditions – PUCCH 1-1 for FD-MIMO (With channelMeasRestriction configured) |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.4.1A FDD CQI Reporting under AWGN conditions - Single CSI Process With interferenceMeasRestriction configured |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.4.1_F FDD CQI Reporting under AWGN conditions - Single CSI Process for CoMP |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.4.2A TDD CQI Reporting under AWGN conditions - Single CSI Process With interferenceMeasRestriction configured |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.4.2_F TDD CQI Reporting under AWGN conditions - Single CSI Process for CoMP |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.5 TDD CQI Reporting under AWGN conditions – PUCCH 1-1 (when csi-SubframeSet – r12 and EIMTA-MainConfigServCell-r12 are configured) |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.6.1 LAA CQI Reporting under AWGN Conditions with Frame Structure Type 3 with FDD as Pcell (PUSCH 3-0) |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.6.2 LAA CQI Reporting under AWGN Conditions with Frame Structure Type 3 with TDD as Pcell (PUSCH 3-0) |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.7.1 LAA CQI Reporting under AWGN Conditions with Frame Structure Type 3 with FDD as Pcell (PUSCH 3-1) |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.7.2 LAA CQI Reporting under AWGN Conditions with Frame Structure Type 3 with TDD as Pcell (PUSCH 3-1) |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.3.1.1.1 FDD CQI Reporting under fading conditions – PUSCH 3-0 |  | SNRs as specified 2% 55% 1.10 BLER 0.05 |  | SNR 0dB 0% 0% 0.01 BLER 0 |  | SNR unchanged  limit unchanged limit unchanged  1.09 as per Table G.5.4-1 BLER limit unchanged |  |
| 9.3.1.1.2 TDD CQI Reporting under fading conditions – PUSCH 3-0 |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  |
| 9.3.1.1.5 TDD CQI Reporting under fading conditions – PUCCH 3-0 (when csi-SubframeSet –r12 is configured) |  | SNRs as specified 2% 55% 1.10 BLER 0.05~0.60 Delta CQI 3 |  | SNR 0dB 0% 0% 0.01 BLER 0 Delta CQI 0 |  | SNR unchanged  limit unchanged limit unchanged  1.09 as per Table G.5.4-1 BLER limit unchanged Delta CQI limit unchanged |  |
| 9.3.1.2.1_D FDD CQI Reporting under fading conditions – PUSCH 3-1 for eDL-MIMO |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  |
| 9.3.1.2.2_D TDD CQI Reporting under fading conditions – PUSCH 3-1 for eDL-MIMO |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  |
| 9.3.1.2.3 FDD CQI Reporting under fading conditions – PUSCH 3-1 for 256QAM in DL |  | SNRs as specified 2% 40 1.10 BLER 0.05 |  | SNR 0dB 0% 0% 0.01 BLER 0 |  | SNR unchanged  limit unchanged limit unchanged  1.09 as per Table G.5.4-1 BLER limit unchanged |  |
| 9.3.1.2.4 TDD CQI Reporting under fading conditions – PUSCH 3-1 for 256QAM in DL |  | Same as 9.3.1.2.3 |  | Same as 9.3.1.2.3 |  | Same as 9.3.1.2.3 |  |
| 9.3.1.2.6 TDD CQI Reporting under fading conditions – PUCCH 3-1 (when csi-SubframeSet –r12 is configured with one CSI process) |  | SNRs as specified 2% 55% 1.02 BLER 0.01 Delta CQI 3 |  | SNR 0dB 0% 0% 0.01 BLER 0 Delta CQI 0 |  | SNR unchanged  limit unchanged limit unchanged  1.01 as per Table G.5.4-1 BLER limit unchanged Delta CQI limit unchanged |  |
| 9.3.1.3.1_E.1 FDD CQI Reporting under fading conditions - PUSCH 3-0 for feICIC (non-MBSFN ABS) |  | SNRs as specified 2% 55% 1.10 BLER 0.01 |  | SNR 0dB 0% 0% 0.01 BLER 0 |  | SNR unchanged  limit unchanged limit unchanged  1.09 as per Table G.5.4-1 BLER limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.3.1.3.2_E.1 TDD CQI Reporting under fading conditions - PUSCH 3-0 for feICIC (non-MBSFN ABS) |  | Same as 9.3.1.3.1_E.1 |  | Same as 9.3.1.3.1_E.1 |  | Same as 9.3.1.3.1_E.1 |  |
| 9.3.2.1.1 FDD CQI Reporting under fading conditions – PUCCH 1-0 |  | SNRs as specified 20% 1.05 BLER 0.02 |  | SNR 0dB 0% 0.01 BLER 0 |  | SNR and BLER limits unchanged  limit unchanged  1.04 as per Table G.5.4-1 BLER limit unchanged |  |
| 9.3.2.1.1_1 FDD CQI Reporting under fading conditions – PUCCH 1-0 |  | Same as 9.3.2.1.1 |  | Same as 9.3.2.1.1 |  | Same as 9.3.2.1.1 |  |
| 9.3.2.1.2 TDD CQI Reporting under fading conditions – PUCCH 1-0 |  | Same as 9.3.2.1.1 |  | Same as 9.3.2.1.1 |  | Same as 9.3.2.1.1 |  |
| 9.3.2.1.2_1 TDD CQI Reporting under fading conditions – PUCCH 1-0 |  | Same as 9.3.2.1.1 |  | Same as 9.3.2.1.1 |  | Same as 9.3.2.1.1 |  |
| 9.3.2.2.1_D FDD CQI Reporting under fading conditions – PUCCH 1-1 for eDL-MIMO |  | Same as 9.3.2.1.1 |  | Same as 9.3.2.1.1 |  | Same as 9.3.2.1.1 |  |
| 9.3.2.2.2_D TDD CQI Reporting under fading conditions – PUCCH 1-1 for eDL-MIMO |  | Same as 9.3.2.1.1 |  | Same as 9.3.2.1.1 |  | Same as 9.3.2.1.1 |  |
| 9.3.3.1.1 FDD CQI Reporting under fading conditions and frequency-selective interference – PUSCH 3-0 |  | Ior and Iot as specified 60% 1.60 |  | Ior, Iot 0dB 20% 0.10 |  | Ior and Iot unchanged  40%, Formula: Min Req’t – Test Tol  1.50, Formula: Min Req’t – Test Tol The effect of AWGN flatness and signal flatness on the requirement was derived by simulation. AWGN flatness / signal flatness and the statistical effect of a finite test time both affect the T-put result. The Test Tolerance comprises two quantities: 1. Effect of AWGN flatness and signal flatness, derived by simulation 2. Statistical effect as per Table G.5.4-1 Items 1 and 2 are assumed to be uncorrelated so can be root sum squared T-put requirement Test Tol = SQRT (AWGN flatness and signal flatness effect 2 + Statistical effect 2) AWGN flatness and signal flatness effect 0.1, Statistical effect 0.01, giving overall effect 0.10 |  |
| 9.3.3.1.2 TDD CQI Reporting under fading conditions and frequency-selective interference – PUSCH 3-0 |  | Same as 9.3.3.1.1 |  | Same as 9.3.3.1.1 |  | Same as 9.3.3.1.1 |  |
| 9.3.4.1.1 FDD CQI Reporting under fading conditions – PUSCH 2-0 |  | SNRs as specified Limits as in the Test Procedure 1.2 |  | 0.01 |  | SNR unchanged  1.19 as per Table G.5.4-1 |  |
| 9.3.4.1.2 TDD CQI Reporting under fading conditions – PUSCH 2-0 |  | Same as 9.3.4.1.1 |  | Same as 9.3.4.1.1 |  | Same as 9.3.4.1.1 |  |
| 9.3.4.2.1 FDD CQI Reporting under fading conditions – PUCCH 2-0 |  | SNRs as specified Limits as in the Test Procedure 1.15 |  | 0.01 |  | SNR unchanged  1.14 as per Table G.5.4-1 |  |
| 9.3.4.2.2 TDD CQI Reporting under fading conditions – PUCCH 2-0 |  | Same as 9.3.4.2.1 |  | Same as 9.3.4.2.1 |  | Same as 9.3.4.2.1 |  |
| 9.3.5.1.1 FDD CQI Reporting under fading conditions - PUCCH 1-0 - Enhanced Performance Requirement Type A |  | Noc: -98dBm/15kHz DIP 1: -0.41dB SINR: -2.00dB 1.8 BLER 0.02 |  | 0dB +0.03dB +0.03dB 0.01 |  | Noc: -98dBm/15kHz DIP 1: -0.38dB SINR: -1.97dB 1.79 as per Table G.5.4-1 BLER limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.3.5.1.2 TDD CQI Reporting under fading conditions - PUCCH 1-0 - Enhanced Performance Requirement Type A |  | Same as 9.3.5.1.1 |  | Same as 9.3.5.1.1 |  | Same as 9.3.5.1.1 |  |
| 9.3.5.2.1 FDD CQI Reporting under fading conditions - PUCCH 1-1 - Enhanced Performance Requirement Type A |  | Same as 9.3.5.1.1 Same as 9.3.5.1.1 |  | Same as 9.3.5.1.1 |  | Same as 9.3.5.1.1 |  |
| 9.3.5.2.2 TDD CQI Reporting under fading conditions - PUCCH 1-1 - Enhanced Performance Requirement Type A |  | Same as 9.3.5.1.1 |  | Same as 9.3.5.1.1 |  |  |  |
| 9.3.6.1_F FDD CQI Reporting under fading conditions Single CSI processes for CoMP |  | 1.02 |  | 0.01 |  |  1.01 as per Table G.5.4-1 |  |
| 9.3.6.1_F.2 FDD CQI Reporting under fading conditions Three CSI processes for CoMP |  | Same as 9.3.6.1_F.1 |  | Same as 9.3.6.1_F.1 |  | Same as 9.3.6.1_F.1 |  |
| 9.3.6.1_F.3 FDD CQI Reporting under fading conditions Four CSI processes for CoMP |  | Same as 9.3.6.1_F.1 |  | Same as 9.3.6.1_F.1 |  | Same as 9.3.6.1_F.1 |  |
| 9.3.6.2_F.1 TDD CQI Reporting under fading conditions Single CSI processes for CoMP |  | 1.02 |  | 0.01 |  |  1.01 as per Table G.5.4-1 |  |
| 9.3.6.2_F.2 TDD CQI Reporting under fading conditions Three CSI processes for CoMP |  | Same as 9.3.6.2_F.1 |  | Same as 9.3.6.2_F.1 |  | Same as 9.3.6.2_F.1 |  |
| 9.3.6.2_F.3 TDD CQI Reporting under fading conditions Four CSI processes for CoMP |  | Same as 9.3.6.2_F.1 |  | Same as 9.3.6.2_F.1 |  | Same as 9.3.6.2_F.1 |  |
| 9.3.7.1 FDD CQI Reporting under fading conditions - PUSCH 3-2 for eDL MIMO Enhancement |  | SNRs as specified Limits as in the Test Procedure Noc: -98dBm/15kHz 1.05 1.15 |  | 0dB 0.01 0.01 |  | SNR unchanged Noc: -98dBm/15kHz 1.04 as per Table G.5.4-1 1.14 as per Table G.5.4-1 The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.3.7.2 TDD CQI Reporting under fading conditions - PUSCH 3-2 for eDL MIMO Enhancement |  | Same as 9.3.7.1 |  | Same as 9.3.7.1 |  | Same as 9.3.7.1 |  |
| 9.3.8.1.1 |  | Noc: -98dBm/15kHz SNR: +8.34dB Cell 2 Ês/Noc: +3.28 dB Cell 3 Ês/Noc: +0.74 dB 0.925 |  | 0 dB +0.63 dB +0.8 dB +0.8 dB 0.01 |  | Noc: -98dBm/15kHz SNR: +8.97dB Cell 2 Ês/Noc: +4.08 dB Cell 3 Ês/Noc: +1.54 dB 0.915 as per Table G.5.4-1 The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.3.8.1.2 |  | Same as 9.3.8.1.1 |  | Same as 9.3.8.1.1 |  | Same as 9.3.8.1.1 |  |
| 9.3.8.2.1 |  | Noc: -98dBm/15kHz SNR: +8.34dB Cell 2 Ês/Noc: +3.28 dB Cell 3 Ês/Noc: +0.74 dB 0.925 |  | 0 dB +0.63 dB +0.8 dB +0.8 dB 0.01 |  | Noc: -98dBm/15kHz SNR: +8.97dB Cell 2 Ês/Noc: +4.08 dB Cell 3 Ês/Noc: +1.54 dB 0.915 as per Table G.5.4-1 The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.3.8.2.2 |  | Same as 9.3.8.2.1 |  | Same as 9.3.8.2.1 |  | Same as 9.3.8.2.1 |  |
| 9.3.8.3.1 FDD CQI Reporting under fading conditions - PUCCH 1-1 (CSI Reference Symbol) TM10 with TM9 interference - Enhanced Receiver Type B |  | Noc: -98dBm/15kHz SNR: +8.34dB Cell 2 Ês/Noc: +3.28 dB Cell 3 Ês/Noc: +0.74 dB 0.925 |  | 0 dB +0.63 dB +0.8 dB +0.8 dB 0.01 |  | Noc: -98dBm/15kHz SNR: +8.97dB Cell 2 Ês/Noc: +4.08 dB Cell 3 Ês/Noc: +1.54 dB  0.915 as per Table G.5.4-1 |  |
| 9.3.8.3.2 TDD CQI Reporting under fading conditions - PUCCH 1-1 (CSI Reference Symbol) TM10 with TM9 interference - Enhanced Receiver Type B |  | Same as 9.3.8.3.1 |  | Same as 9.3.8.3.1 |  | Same as 9.3.8.3.1 |  |
| 9.4.1.1.1 FDD PMI Reporting – PUSCH 3-1 (Single PMI) |  | 1.10 |  | 0.01 |  |  1.09 as per Table G.5.4-1 |  |
| 9.4.1.1.2 TDD PMI Reporting – PUSCH 3-1 (Single PMI) |  | Same as 9.4.1.1.1 |  | Same as 9.4.1.1.1 |  | Same as 9.4.1.1.1 |  |
| 9.4.1.2.1 FDD PMI Reporting – PUCCH 2-1 (Single PMI) |  | Noc as specified in test procedure 1.2 |  | 0.01 |  | Noc unchanged Ior/ Noc=SNRrnd is a result of an approach according to G.5.2 and is reused unchanged as setting in procedure step 3.  1.19 as per Table G.5.4-1 |  |
| 9.4.1.2.2 TDD PMI Reporting – PUCCH 2-1 (Single PMI) |  | Same as 9.4.1.2.1 |  | Same as 9.4.1.2.1 |  | Same as 9.4.1.2.1 |  |
| 9.4.1.3.1_D FDD Reporting of PMI – PUSCH 3-1 (Single PMI) for eDL-MIMO |  | Same as 9.4.1.2.1 |  | Same as 9.4.1.2.1 |  | Same as 9.4.1.2.1 |  |
| 9.4.1.3.2_D TDD Reporting of PMI – PUSCH 3-1 (Single PMI) for eDL-MIMO |  | Same as 9.4.1.2.1 |  | Same as 9.4.1.2.1 |  | Same as 9.4.1.2.1 |  |
| 9.4.1.3.3 FDD PMI Reporting with 12Tx Class A codebook – PUSCH 3-1 (Single PMI) for FD-MIMO |  | Noc as specified in test procedure 2.50 |  | 0.01 |  | Noc unchanged Ior/ Noc=SNRrnd is a result of an approach according to G.5.2 and is used unchanged.  2.49 as per Table G.5.4-1 |  |
| 9.4.1.3.4 TDD PMI Reporting with 12Tx Class A codebook – PUSCH 3-1 (Single PMI) for FD-MIMO |  | Same as 9.4.1.3.3 |  | Same as 9.4.1.3.3 |  | Same as 9.4.1.3.3 |  |
| 9.4.1.4.1 FDD PMI Reporting with 4Tx enhanced codebook – PUCCH 1-1 (Single PMI) for eDL MIMO Enhancement 9.6.1.3.2, TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for FDD PCell (3DL CA) |  | Noc: -98dBm/15kHz 1.8 |  | 0.01 |  | Noc: -98dBm/15kHz 1.79 as per Table G.5.4-1 The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.4.1.4.2 TDD PMI Reporting with 4Tx enhanced codebook – PUCCH 1-1 (Single PMI) for eDL MIMO Enhancement 9.6.1.4.2, TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for TDD PCell (3DL CA) |  | Same as 9.4.1.4.1 |  | Same as 9.4.1.4.1 |  | Same as 9.4.1.4.1 |  |
| 9.4.1.4.3 FDD PMI Reporting with Class B alternative codebook – PUCCH 1-1 for FD-MIMO |  | Same as 9.4.1.3.3 |  | Same as 9.4.1.3.3 |  | Same as 9.4.1.3.3 |  |
| 9.4.1.4.4 TDD PMI Reporting with Class B alternative codebook – PUCCH 1-1 for FD-MIMO |  | Same as 9.4.1.3.3 |  | Same as 9.4.1.3.3 |  | Same as 9.4.1.3.3 |  |
| 9.4.2.1.1 FDD PMI Reporting – PUSCH 1-2 (Multiple PMI) |  | Noc as specified in test procedure 1.20 |  | 0.01 |  | Noc unchanged Ior/ Noc=SNRrnd is a result of an approach according to G.5.2 and is reused unchanged as setting in procedure step 3.  1.19 as per Table G.5.4-1 |  |
| 9.4.2.1.1_1 FDD PMI Reporting – PUSCH 1-2 (Multiple PMI) |  | Same as 9.4.2.1.1 |  | Same as 9.4.2.1.1 |  | Same as 9.4.2.1.1 |  |
| 9.4.2.1.2 TDD PMI Reporting – PUSCH 1-2 (Multiple PMI) |  | Same as 9.4.2.1.1 |  | Same as 9.4.2.1.1 |  | Same as 9.4.2.1.1 |  |
| 9.4.2.1.2_1 TDD PMI Reporting – PUSCH 1-2 (Multiple PMI) |  | Same as in 9.4.2.1.1 |  | Same as in 9.4.2.1.1 |  | Same as in 9.4.2.1.1 |  |
| 9.4.2.2.1 FDD PMI Reporting – PUSCH 2-2 (Multiple PMI) |  | Same as in 9.4.2.1.1 |  | Same as in 9.4.2.1.1 |  | Same as in 9.4.2.1.1 |  |
| 9.4.2.2.2 TDD PMI Reporting – PUSCH 2-2 (Multiple PMI) |  | Noc as specified in test procedure 1.15 |  | 0.01 |  | Noc unchanged Ior/ Noc=SNRrnd is a result of an approach according to G.5.2 and is reused unchanged as setting in procedure step 3.  1.14 as per Table G.5.4-1 |  |
| 9.4.2.3.1_D FDD PMI Reporting – PUSCH 1-2 (Multiple PMI) for eDL-MIMO |  | Noc as specified in test procedure 1.30 |  | 0.01 |  | Noc unchanged Ior/ Noc=SNRrnd is a result of an approach according to G.5.2 and is reused unchanged as setting in procedure step 3.  1.29 as per Table G.5.4-1 |  |
| 9.4.2.3.2_D TDD PMI Reporting – PUSCH 1-2 (Multiple PMI) for eDL-MIMO |  | Noc as specified in test procedure 3.50 |  | 0.01 |  | Noc unchanged Ior/ Noc=SNRrnd is a result of an approach according to G.5.2 and is reused unchanged as setting in procedure step 3.  3.49 as per Table G.5.4-1 |  |
| 9.4.2.3.3 FDD PMI Reporting with 4Tx enhanced codebook - PUSCH 1-2 (Multiple PMI) for eDL-MIMO Enhancement |  | Noc: -98dBm/15kHz 1.2 |  | 0.01 |  | Noc: -98dBm/15kHz 1.19 as per Table G.5.4-1 The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.4.2.3.4 TDD PMI Reporting with 4Tx enhanced codebook - PUSCH 1-2 (Multiple PMI) for eDL-MIMO Enhancement |  | Same as 9.4.2.3.3 |  | Same as 9.4.2.3.3 |  | Same as 9.4.2.3.3 |  |
| 9.4.2.3.5 FDD PMI Reporting with Class A 16Tx codebook - PUSCH 1-2 (Multiple PMI) for FD-MIMO |  | Same as 9.4.1.3.3 |  | Same as 9.4.1.3.3 |  | Same as 9.4.1.3.3 |  |
| 9.4.2.3.6 TDD PMI Reporting with Class A 16Tx codebook - PUSCH 1-2 (Multiple PMI) for FD-MIMO |  | Same as 9.4.1.3.3 |  | Same as 9.4.1.3.3 |  | Same as 9.4.1.3.3 |  |
| 9.5.1.1 FDD RI Reporting– PUCCH 1-1 |  | SNRs as specified Test 1: 21.00 Test 2: 11.05 Test 3: 21.10 |  | SNR 0dB 20.01 10.01 20.01 |  | SNR unchanged 2 0.99 as per Table G.5.4-1 1 1.04 as per Table G.5.4-1 2 1.09 as per Table G.5.4-1 |  |
| 9.5.1.1_1 FDD RI Reporting– PUCCH 1-1 (Release 10) |  | SNRs as specified Test 1: 21.00 Test 2: 11.05 Test 3: 10.90, 21.10 |  | SNR 0dB 20.01 10.01 20.01 |  | SNR unchanged 2 0.99 as per Table G.5.4-1 1 1.04 as per Table G.5.4-1 1 0.89, 2 1.09 as per Table G.5.4-1 |  |
| 9.5.1.1_2 FDD RI Reporting– PUCCH 1-1 (Release 11) |  | SNRs as specified Test 1: 21.00 Test 2: 11.05 Test 3: 10.90 |  | SNR 0dB 20.01 10.01 10.01 |  | SNR unchanged 2 0.99 as per Table G.5.4-1 1 1.04 as per Table G.5.4-1 1 0.89 as per Table G.5.4-1 |  |
| 9.5.1.2 TDD RI Reporting– PUSCH 3-1 |  | Same as 9.5.1.1 |  | Same as 9.5.1.1 |  | Same as 9.5.1.1 |  |
| 9.5.1.2_1 TDD RI Reporting– PUSCH 3-1 (Release 10) |  | Same as 9.5.1.1_1 |  | Same as 9.5.1.1_1 |  | Same as 9.5.1.1_1 |  |
| 9.5.1.2_2 TDD RI Reporting– PUSCH 3-1 (Release 11) |  | Same as 9.5.1.1_2 |  | Same as 9.5.1.1_2 |  | Same as 9.5.1.1_2 |  |
| 9.5.2.1_D FDD RI Reporting – PUCCH 1-1 for eDL-MIMO |  | SNR as specified Test 1: 21.00 Test 2: 11.05 Test 3: 10.9 |  | SNR 0dB 20.01 10.01 20.01 |  | SNR unchanged 2 0.99 as per Table G.5.4-1 1 1.04 as per Table G.5.4-1 10.89 as per Table G.5.4-1 |  |
| 9.5.2.2_D TDD RI Reporting – PUCCH 1-1 for eDL-MIMO |  | SNR as specified Test 1: 21.00 Test 2: 11.05 Test 3: 10.9 |  | SNR 0dB 20.01 10.01 20.01 |  | SNR unchanged 2 0.99 as per Table G.5.4-1 1 1.04 as per Table G.5.4-1 10.89 as per Table G.5.4-1 |  |
| 9.5.3.1_C.1 FDD RI Reporting – PUCCH 1-0 for eICIC (non-MBSFN ABS) |  | SNRs as specified Test 1: 10.90 Test 2: 11.05 |  | SNR 0dB 10.01 10.01 |  | SNR unchanged 1 0.89 as per Table G.5.4-1 1 1.04 as per Table G.5.4-1 The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.5.3.2_C.1 TDD RI Reporting – PUCCH 1-0 for eICIC (non-MBSFN ABS) |  | SNRs as specified Test 1: 10.90 Test 2: 11.05 |  | SNR 0dB 10.01 10.01 |  | SNR unchanged 1 0.89 as per Table G.5.4-1 1 1.04 as per Table G.5.4-1 The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.5.4.1_E.1 FDD RI Reporting - PUCCH 1-0 for feICIC (non-MBSFN ABS) |  | SNR as specified Test 1: 21.05 Test 2: 11.05 Test 3: 10.9 |  | SNR 0dB 20.01 10.01 10.01 |  | SNR unchanged 2 1.04 as per Table G.5.4-1 1 1.04 as per Table G.5.4-1 10.89 as per Table G.5.4-1 The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.5.4.2_E.1 TDD RI Reporting - PUCCH 1-0 for feICIC (non-MBSFN ABS) |  | Same as 9.5.4.1_E.1 |  | Same as 9.5.4.1_E.1 |  | Same as 9.5.4.1_E.1 |  |
| 9.5.5.1_F.1 FDD RI Reporting with Single CSI process for CoMP |  | SNRs as specified Test 1: 21.00 Test 2: 11.00 |  | SNR 0dB 20.01 10.01 |  | SNR unchanged 2 0.99 as per Table G.5.4-1 1 0.99 as per Table G.5.4-1 |  |
| 9.5.5.1_F.2 FDD RI Reporting with Multiple CSI processes for CoMP |  | SNRs as specified Test 1: 21.00 Test 2: 11.00 |  | SNR 20dB 20.01 10.01 |  | SNR unchanged 2 0.99 as per Table G.5.4-1 1 0.99 as per Table G.5.4-1 |  |
| 9.5.5.2_F.1 TDD RI Reporting with Single CSI process for CoMP |  | Same as 9.5.5.1_F.1 |  | Same as 9.5.5.1_F.1 |  | Same as 9.5.5.1_F.1 |  |
| 9.5.5.2_F.2 TDD RI Reporting with Multiple CSI processes for CoMP |  | Same as 9.5.5.2_F.1 |  | Same as 9.5.5.2_F.1 |  | Same as 9.5.5.2_F.1 |  |
| 9.6.1.1_A.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 for CA (2DL CA) |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR ratios each have an uncertainty of ±0.3 dB. For uncorrelated uncertainties, the overall uncertainty in the SNR difference between any two CCs would be SQRT (0.32 + 0.32) = 0.42dB. This could widen the Median CQI difference, but the uncertainty is significantly less than one reported CQI step. Relaxing the allowed Median CQI difference would relax the test excessively, so in this case the Test requirement should be left unchanged. |  |
| 9.6.1.1_A.2 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 (3DL CA) |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | Same as 9.6.1.1_A.1 |  |
| 9.6.1.1_A.4 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 (5DL CA) |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | Same as 9.6.1.1_A.1 |  |
| 9.6.1.1_A.5 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 (6DL CA) |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | Same as 9.6.1.1_A.1 |  |
| 9.6.1.1_A.6 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 (7DL CA) |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | Same as 9.6.1.1_A.1 |  |
| 9.6.1.2_A.1 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 for 2DL CA |  | Same as 9.6.1.1_A.1 |  | Same as 9.6.1.1_A.1 |  | Same as 9.6.1.1_A.1 |  |
| 9.6.1.2_A.2 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 (3DL CA) |  | Same as 9.6.1.1_A.2 |  | Same as 9.6.1.1_A.2 |  | Same as 9.6.1.1_A.2 |  |
| 9.6.1.2_A.3 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 (4DL CA) |  | Same as 9.6.1.1_A.3 |  | Same as 9.6.1.1_A.3 |  | Same as 9.6.1.1_A.3 |  |
| 9.6.1.2_A.4 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 (5DL CA) |  | Same as 9.6.1.1_A.4 |  | Same as 9.6.1.1_A.4 |  | Same as 9.6.1.1_A.4 |  |
| 9.6.1.2_A.5 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 (6DL CA) |  | Same as 9.6.1.1_A.5 |  | Same as 9.6.1.1_A.5 |  | Same as 9.6.1.1_A.5 |  |
| 9.6.1.2_A.6 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 (7DL CA) |  | Same as 9.6.1.1_A.6 |  | Same as 9.6.1.1_A.6 |  | Same as 9.6.1.1_A.6 |  |
| 9.6.1.3.1 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for FDD PCell (2DL CA) |  | Same as 9.6.1.1_A.1 |  | Same as 9.6.1.1_A.1 |  | Same as 9.6.1.1_A.1 The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.6.1.3.2 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for FDD PCell (3DL CA) |  | Same as 9.6.1.1_A.2 |  | Same as 9.6.1.1_A.2 |  | Same as 9.6.1.1_A.2 The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.6.1.3.3 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for FDD PCell (4DL CA) |  | Same as 9.6.1.1_A.1 |  | Same as 9.6.1.1_A.1 |  | Same as 9.6.1.1_A.1 |  |
| 9.6.1.3.4 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for FDD PCell (5DL CA) |  | Same as 9.6.1.1_A.1 |  | Same as 9.6.1.1_A.1 |  | Same as 9.6.1.1_A.1 |  |
| 9.6.1.4.1 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for TDD PCell (2DL CA) |  | Same as 9.6.1.1_A.1 |  | Same as 9.6.1.1_A.1 |  | Same as 9.6.1.1_A.1 The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.6.1.4.2 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for TDD PCell (3DL CA) |  | Same as 9.6.1.1_A.2 |  | Same as 9.6.1.1_A.2 |  | Same as 9.6.1.1_A.2 The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.6.1.4.3 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for TDD PCell (4DL CA) |  | Same as 9.6.1.1_A.1 |  | Same as 9.6.1.1_A.1 |  | Same as 9.6.1.1_A.1 |  |
| 9.6.1.4.4 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for TDD PCell (5DL CA) |  | Same as 9.6.1.1_A.1 |  | Same as 9.6.1.1_A.1 |  | Same as 9.6.1.1_A.1 |  |
| 9.7.1.1 FDD and Half duplex FDD CQI reporting definition under AWGN conditions for UE category 0 |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.7.1.2 TDD CQI reporting definition under AWGN conditions for UE category 0 |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.7.2.1 FDD and Half duplex FDD CQI reporting definition under fading conditions for UE category 0 |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  |
| 9.7.2.2 TDD CQI reporting definition under fading conditions for UE category 0 |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  |
| 9.7_1.1.1 FDD CQI reporting under AWGN conditions - PUCCH 1-0 for UE category 1bis |  | Same as 9.2.1.1 |  | Same as 9.2.1.1 |  | Same as 9.2.1.1 |  |
| 9.7_1.1.2 TDD CQI reporting under AWGN conditions - PUCCH 1-0 for UE category 1bis |  | Same as 9.2.1.2 |  | Same as 9.2.1.2 |  | Same as 9.2.1.2 |  |
| 9.7_1.1.3 FDD CQI reporting under fading conditions - PUSCH 3-0 for UE category 1bis |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  |
| 9.7_1.1.4 TDD CQI reporting under fading conditions - PUSCH 3-0 for UE category 1bis |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  |
| 9.8.1.1 FDD and Half duplex FDD CQI reporting definition under AWGN conditions for UE category M1 |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.8.1.2 TDD CQI reporting definition under AWGN conditions for UE category M1 |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.8.2.1 FDD and Half duplex FDD CQI reporting definition under fading conditions for UE category M1 |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  |
| 9.8.2.2 TDD CQI reporting definition under fading conditions for UE category M1 |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  |
| 9.9.1.1.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 with Rank 1 1x4 |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.9.1.1.2 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 with Rank 1 1x4 |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.9.1.2.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-1 with rank 2 4x4 |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.9.1.2.2 TDD CQI Reporting under AWGN conditions – PUCCH 1-1 with rank 2 8x4 |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.9.1.3.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-1 with rank 4 4x4 |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.9.1.3.2 TDD CQI Reporting under AWGN conditions – PUCCH 1-1 with rank 4 4x4 |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.9.1.4.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-1 with rank 3 4x4 |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.9.1.4.2 TDD CQI Reporting under AWGN conditions – PUCCH 1-1 with rank 3 8x4 |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.9.2.1.1 FDD CQI Reporting under fading conditions - PUCCH 1-0 - Enhanced Performance Requirement Type A 1x4 |  | Noc: -98dBm/15kHz DIP 1: -0.41dB SINR: -4.00dB 1.8 BLER 0.02 |  | 0dB +0.03dB +0.03dB 0.01 |  | Noc: -98dBm/15kHz DIP 1: -0.38dB SINR: -3.97dB 1.79 as per Table G.5.4-1 BLER limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.9.2.1.2 TDD CQI Reporting under fading conditions - PUCCH 1-0 - Enhanced Performance Requirement Type A 1x4 |  | Same as 9.9.2.1.1 |  | Same as 9.9.2.1.1 |  | Same as 9.9.2.1.1 |  |
| 9.9.2.2.1 FDD CQI Reporting under fading conditions - PUCCH 1-1 - Enhanced Performance Requirement Type A 2x4 |  | Noc: -98dBm/15kHz DIP 1: -0.41dB SINR: -4.00dB 1.8 BLER 0.02 |  | 0dB +0.03dB +0.03dB 0.01 |  | Noc: -98dBm/15kHz DIP 1: -0.38dB SINR: -3.97dB 1.79 as per Table G.5.4-1 BLER limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.9.2.2.2 TDD CQI Reporting under fading conditions - PUCCH 1-1 - Enhanced Performance Requirement Type A 2x4 |  | Same as 9.9.2.2.1 |  | Same as 9.9.2.2.1 |  | Same as 9.9.2.2.1 |  |
| 9.9.3.1.1 TDD PMI Reporting – PUSCH 3-1 (Single PMI) 8x4 |  | Noc as specified in test procedure 2.50 |  | 0.01 |  | Noc unchanged Ior/ Noc=SNRrnd is a result of test procedure step 2 and is used as setting in test procedure step 3. 2.49 as per Table G.5.4-1 |  |
| 9.9.4.1.1 FDD RI Reporting– PUCCH 1-1 4x4 |  | SNRs as specified Test 1: 21.00 Test 2: 11.05 Test 3: 10.9 Test 4: 2 1.10 |  | SNR 0dB 20.01 10.01 10.01 20.01 |  | SNR unchanged 2 0.99 as per Table G.5.4-1 1 1.04 as per Table G.5.4-1 1 0.89 as per Table G.5.4-1 2 1.09 as per Table G.5.4-1 |  |
| 9.9.4.1.2 TDD RI Reporting– PUSCH 3-1 4x4 |  | SNRs as specified Test 1: 21.00 Test 2: 11.05 Test 3: 10.9 Test 4: 2 1.10 |  | SNR 0dB 20.01 10.01 10.01 20.01 |  | SNR unchanged 2 0.99 as per Table G.5.4-1 1 1.04 as per Table G.5.4-1 1 0.89 as per Table G.5.4-1 2 1.09 as per Table G.5.4-1 |  |
| 9.9.4.2.1 FDD RI Reporting- PUCCH 1-1 for eDL-MIMO 1x4 |  | SNRs as specified Test 1: 21.00 Test 2: 11.05 Test 3: 10.9 Test 4: 2 1.10 |  | SNR 0dB 20.01 10.01 10.01 20.01 |  | SNR unchanged 2 0.99 as per Table G.5.4-1 1 1.04 as per Table G.5.4-1 1 0.89 as per Table G.5.4-1 2 1.09 as per Table G.5.4-1 |  |
| 9.9.4.2.2 TDD RI Reporting- PUCCH 1-1 for eDL-MIMO 1x4 |  | SNRs as specified Test 1: 21.00 Test 2: 11.05 Test 3: 10.9 Test 4: 2 1.10 |  | SNR 0dB 20.01 10.01 10.01 20.01 |  | SNR unchanged 2 0.99 as per Table G.5.4-1 1 1.04 as per Table G.5.4-1 1 0.89 as per Table G.5.4-1 2 1.09 as per Table G.5.4-1 |  |
| 9.10.1.1 FDD CSI-RS Resource Indicator Reporting – PUSCH 3-1 |  | SNRs as specified Test 1-1: 1.2 Test 1-2: 1.2 Test 1-3: 1.3 Test 1-4:  1.35 |  | SNR 0dB 0.01 |  | SNR unchanged  1.19 as per Table G.5.4-1  1.19 as per Table G.5.4-1  1.29 as per Table G.5.4-1  1.34 as per Table G.5.4-1 |  |
| 9.10.1.2 TDD CSI-RS Resource Indicator Reporting – PUSCH 3-1 |  | SNRs as specified Test 1-1: 1.2 Test 1-2: 1.2 Test 1-3: 1.3 Test 1-4:  1.35 |  | SNR 0dB 0.01 |  | SNR unchanged  1.19 as per Table G.5.4-1  1.19 as per Table G.5.4-1  1.29 as per Table G.5.4-1  1.34 as per Table G.5.4-1 |  |
| 9.12.1.1 FDD CQI Reporting under fading conditions (with cell-specific reference symbols configured for UE supporting short TTI) |  | SNRs as specified 20% 1.05 BLER 0.02 |  | SNR 0dB 0% 0.01 BLER 0 |  | SNR and BLER limits unchanged  limit unchanged  1.04 as per Table G.5.4-1 BLER limit unchanged |  |
| 9.12.1.1 TDD CQI Reporting under fading conditions (with cell-specific reference symbols configured for UE supporting short TTI) |  | SNRs as specified 20% 1.05 BLER 0.02 |  | SNR 0dB 0% 0.01 BLER 0 |  | SNR and BLER limits unchanged  limit unchanged  1.04 as per Table G.5.4-1 BLER limit unchanged |  |
| 9.12.2.1 FDD CQI Reporting under fading conditions (with CSI reference symbols configured for UE supporting short TTI) |  | SNRs as specified 20% 1.05 BLER [0.02] |  | SNR 0dB 0% 0.01 BLER 0 |  | SNR and BLER limits unchanged  limit unchanged  1.04 as per Table G.5.4-1 BLER limit unchanged |  |
| 9.12.2.2 FDD CQI Reporting under fading conditions (with CSI reference symbols configured for UE supporting short TTI) |  | SNRs as specified 20% 1.05 BLER [0.02] |  | SNR 0dB 0% 0.01 BLER 0 |  | SNR and BLER limits unchanged  limit unchanged  1.04 as per Table G.5.4-1 BLER limit unchanged |  |

Annex G (normative):
Statistical Testing

# G.1	General

FFS.

# G.2	Statistical testing of receiver characteristics

## G.2.1	General

The test of receiver characteristics is two fold.

1.	A signal or a combination of signals is offered to the RX port(s) of the receiver.

2.	The ability of the receiver to demodulate /decode this signal is verified by measuring the throughput.

In (2) is the statistical aspect of the test and is treated here.

The minimum requirement for all receiver tests is >95% of the maximum throughput.

All receiver tests are performed in static propagation conditions. No fading conditions are applied.

## G.2.2	Mapping throughput to error ratio

a)	The measured information bit throughput R is defined as the sum (in kilobits) of the information bit payloads successfully received during the test interval, divided by the duration of the test interval (in seconds).

b)	In measurement practice the UE indicates successfully received information bit payload by signalling an ACK to the SS.
If payload is received, but damaged and cannot be decoded, the UE signals a NACK.

c)	Only the ACK and NACK signals, not the data bits received, are accessible to the SS.
The number of bits is known in the SS from knowledge of what payload was sent.

d)	For the reference measurement channel, applied for testing, the number of bits is different in different subframes, however in a radio frame it is fixed during one test.

e)	The time in the measurement interval is composed of successfully received subframes (ACK), unsuccessfully received subframes (NACK) and no reception at all (DTX-subframes).

f)	DTX-subframes may occur regularly according the applicable reference measurement channel (regDTX).
In real live networks this is the time when other UEs are served. In TDD these are the UL and special subframes.
regDTX vary from test to test but are fixed within the test.

g)	Additional DTX-subframes occur statistically when the UE is not responding ACK or NACK where it should. (statDTX)
This may happen when the UE was not expecting data or decided that the data were not intended for it.

The pass / fail decision is done by observing the:

-	number of NACKs

-	number of ACKs and

-	number of statDTXs (regDTX is implicitly known to the SS)

The ratio (NACK + statDTX)/(NACK+ statDTX + ACK)is the Error Ratio (ER). Taking into account the time consumed by the ACK, NACK, and DTX-TTIs (regular and statistical), ER can be mapped unambiguously to throughput for any single reference measurement channel test.

## G.2.3	Design of the test

The test is defined by the following design principles (see clause G.7, Theory….):

1.	The early decision concept is applied.

2.	A second limit is introduced: Bad DUT factor M>1

3.	To decide the test pass:

Supplier risk is applied based on the Bad DUT quality

To decide the test fail

Customer Risk is applied based on the specified DUT quality

The test is defined by the following parameters:

1.	Limit ER = 0.05 (Throughput limit = 95%)

2.	Bad DUT factor M=1.5 (selectivity)

3.	Confidence level CL = 95% (for specified DUT and Bad DUT-quality)

## G.2.4	Numerical definition of the pass fail limits

Table G.2.4-1: pass fail limits

| ne | nsp | nsf | ne | nsp | nsf | ne | nsp | nsf | ne | nsp | nsf |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 67 | NA | 37 | 715 | 477 | 74 | 1290 | 1093 | 111 | 1847 | 1739 |
| 1 | 67 | NA | 38 | 731 | 493 | 75 | 1306 | 1110 | 112 | 1862 | 1756 |
| 2 | 95 | NA | 39 | 747 | 509 | 76 | 1321 | 1128 | 113 | 1877 | 1774 |
| 3 | 119 | NA | 40 | 763 | 525 | 77 | 1336 | 1145 | 114 | 1891 | 1792 |
| 4 | 141 | NA | 41 | 779 | 541 | 78 | 1351 | 1162 | 115 | 1906 | 1809 |
| 5 | 162 | NA | 42 | 795 | 557 | 79 | 1366 | 1179 | 116 | 1921 | 1827 |
| 6 | 183 | NA | 43 | 810 | 573 | 80 | 1382 | 1197 | 117 | 1936 | 1845 |
| 7 | 203 | NA | 44 | 826 | 590 | 81 | 1397 | 1214 | 118 | 1951 | 1863 |
| 8 | 222 | NA | 45 | 842 | 606 | 82 | 1412 | 1231 | 119 | 1966 | 1880 |
| 9 | 241 | 67 | 46 | 858 | 622 | 83 | 1427 | 1248 | 120 | 1981 | 1898 |
| 10 | 259 | 80 | 47 | 873 | 639 | 84 | 1442 | 1266 | 121 | 1995 | 1916 |
| 11 | 278 | 92 | 48 | 889 | 655 | 85 | 1457 | 1283 | 122 | 2010 | 1934 |
| 12 | 296 | 105 | 49 | 905 | 672 | 86 | 1472 | 1300 | 123 | 2025 | 1951 |
| 13 | 314 | 118 | 50 | 920 | 688 | 87 | 1487 | 1318 | 124 | 2040 | 1969 |
| 14 | 332 | 131 | 51 | 936 | 705 | 88 | 1503 | 1335 | 125 | 2055 | 1987 |
| 15 | 349 | 145 | 52 | 952 | 721 | 89 | 1518 | 1353 | 126 | 2069 | 2005 |
| 16 | 367 | 159 | 53 | 967 | 738 | 90 | 1533 | 1370 | 127 | 2084 | 2023 |
| 17 | 384 | 173 | 54 | 983 | 755 | 91 | 1548 | 1387 | 128 | 2099 | 2041 |
| 18 | 401 | 187 | 55 | 998 | 771 | 92 | 1563 | 1405 | 129 | 2114 | 2058 |
| 19 | 419 | 201 | 56 | 1014 | 788 | 93 | 1578 | 1422 | 130 | 2128 | 2076 |
| 20 | 436 | 216 | 57 | 1029 | 805 | 94 | 1593 | 1440 | 131 | 2143 | 2094 |
| 21 | 453 | 230 | 58 | 1045 | 822 | 95 | 1608 | 1457 | 132 | 2158 | 2112 |
| 22 | 469 | 245 | 59 | 1060 | 838 | 96 | 1623 | 1475 | 133 | 2173 | 2130 |
| 23 | 486 | 260 | 60 | 1076 | 855 | 97 | 1638 | 1492 | 134 | 2187 | 2148 |
| 24 | 503 | 275 | 61 | 1091 | 872 | 98 | 1653 | 1510 | 135 | 2202 | 2166 |
| 25 | 520 | 290 | 62 | 1107 | 889 | 99 | 1668 | 1527 | 136 | 2217 | 2183 |
| 26 | 536 | 305 | 63 | 1122 | 906 | 100 | 1683 | 1545 | 137 | 2232 | 2201 |
| 27 | 553 | 320 | 64 | 1137 | 923 | 101 | 1698 | 1562 | 138 | 2246 | 2219 |
| 28 | 569 | 335 | 65 | 1153 | 940 | 102 | 1713 | 1580 | 139 | 2261 | 2237 |
| 29 | 585 | 351 | 66 | 1168 | 957 | 103 | 1728 | 1598 | 140 | 2276 | 2255 |
| 30 | 602 | 366 | 67 | 1184 | 974 | 104 | 1742 | 1615 | 141 | 2291 | 2273 |
| 31 | 618 | 382 | 68 | 1199 | 991 | 105 | 1757 | 1633 | 142 | 2305 | 2291 |
| 32 | 634 | 398 | 69 | 1214 | 1008 | 106 | 1772 | 1650 | 143 | 2320 | 2309 |
| 33 | 651 | 413 | 70 | 1229 | 1025 | 107 | 1787 | 1668 | 144 | 2335 | 2327 |
| 34 | 667 | 429 | 71 | 1245 | 1042 | 108 | 1802 | 1686 | 145 | 2349 | 2345 |
| 35 | 683 | 445 | 72 | 1260 | 1059 | 109 | 1817 | 1703 | 146 | 2364 | 2363 |
| 36 | 699 | 461 | 73 | 1275 | 1076 | 110 | 1832 | 1721 | *) note 2 in G.2.5 |  |  |

NOTE 1:	The first column is the number of errors (ne = number of NACK + statDTX)

NOTE 2:	The second column is the number of samples for the pass limit (nsp, ns=Number of Samples= number of NACK + statDTX + ACK)

NOTE 3:	The third column is the number of samples for the fail limit (nsf)

NOTE 4:	The UE could be decided as early pass/fail when at least 67 samples are received. The nsf is set to NA for ne less than 9.

## G.2.5	Pass fail decision rules

The pass fail decision rules apply for a single test, comprising one component in the test vector. The over all Pass /Fail conditions are defined in clause G.2.6and G.2A.6

Having observed   0  errors, pass the test at      67+    samples,                                                 otherwise continue

Having observed   1  error,   pass the test at      67+    samples,                                                 otherwise continue

Having observed   2  errors,  pass the test at      95+    samples,                                                 otherwise continue

Etc. etc.

Having observed 145 errors,  pass the test at  2349+  samples, fail the test at 2345- samples,  otherwise continue

Having observed 146 errors,  pass the test at  2364+  samples, fail the test at 2363- samples.

Where x+ means: x or more,        x- means x or less

NOTE 1:	an ideal DUT passes after 67 samples. The maximum test time is 2364 samples.

NOTE 2:	It is allowed to deviate from the early decision concept  by  postponing  the decision (pass/fail or continue). Postponing the decision to or beyond the end of Table G.2.4-1 requires a pass fail decision against the test limit: pass the DUT for ER<0.0618, otherwise fail.

## G.2.6	Test conditions for receiver tests

Table G.2.6-1: Test conditions for receiver tests

| Test | Statistical independence | Number of components in the test vector, as specified in the test requirements and initial conditions of the applicable test | Over all Pass/Fail condition |
| --- | --- | --- | --- |
| 7.3 Reference sensitivity level | Yes: the inherent receiver noise is assumed to be AWGN | tbd | To pass 7.3 each component in the test vector must pass |
| 7.3F.1 Reference sensitivity level without repetitions for category NB1 | Yes: the inherent receiver noise is assumed to be AWGN | tbd | To pass 7.3F each component in the test vector must pass |
| 7.4 Maximum input level | Unclear: in case, clipping causes errors, errors are data dependent. Statistical independence is assumed. | tbd | To pass 7.4 each component in the test vector must pass |
| 7.4A.3 Maximum input level for CA (inter-band DL CA without UL CA) | Unclear: in case, clipping causes errors, errors are data dependent. Statistical independence is assumed. | tbd | To pass 7.4A.3 each component in the test vector must pass |
| 7.4F Maximum input level for category NB1 | Unclear: in case, clipping causes errors, errors are data dependent. Statistical independence is assumed. | tbd | To pass 7.4f each component in the test vector must pass |
| 7.5 Adjacent Channel Selectivity (ACS) | Unclear: errors are data dependent on the interferers data. Statistical independence is assumed. | tbd | To pass 7.5 each component in the test vector must pass |
| 7.5A.3 Adjacent Channel Selectivity (ACS) for CA (inter-band DL CA without UL CA) | Unclear: errors are data dependent on the interferers data. Statistical independence is assumed. | tbd | To pass 7.5A.3 each component in the test vector must pass |
| 7.5F Adjacent Channel Selectivity (ACS) for category NB1 | Unclear: errors are data dependent on the interferers data. Statistical independence is assumed | 8 | To pass 7.5F each component in the test vector must pass |
| 7.6.1 In-band blocking | Unclear: errors are data dependent on the interferers data. Statistical independence is assumed. | tbd | To pass 7.6.1 each component in the test vector must pass |
| 7.6.1_1 In-band blocking with 4 Rx antenna ports | Unclear: errors are data dependent on the interferers data. Statistical independence is assumed. | tbd | To pass 7.6.1_1 each component in the test vector must pass |
| 7.6.1A.3 In-band blocking for CA (inter-band DL CA without UL CA) | Unclear: errors are data dependent on the interferers data. Statistical independence is assumed. | tbd | To pass 7.6.1A.3 each component in the test vector must pass |
| 7.6.2 Out of-band blocking | yes: it is assumed that the CW interferer causes errors, which are independent and time invariant. | tbd | To pass 7.6.2, all except [tbd] components in the test vector must pass |
| 7.6.2A.3 Out-of-band blocking for CA (inter-band DL CA without UL CA) | yes: it is assumed that the CW interferer causes errors, which are independent and time invariant. | tbd | To pass 7.6.2A.3, all except [tbd] components in the test vector must pass |
| 7.6.3 Narrow band blocking | yes: it is assumed that the CW interferer causes errors, which are independent and time invariant. | tbd | To pass 7.6.3 each component in the test vector must pass |
| 7.6.3A.3 Narrow band blocking for CA (inter-band DL CA without UL CA) | yes: it is assumed that the CW interferer causes errors, which are independent and time invariant. | tbd | To pass 7.6.3A.3 each component in the test vector must pass |
| 7.7 Spurious response | yes: it is assumed that the CW interferer causes errors, which are independent and time invariant. | tbd | To pass 7.7 each component in the test vector must pass |
| 7.7A.3 Spurious response for CA (inter-band DL CA without UL CA) | yes: it is assumed that the CW interferer causes errors, which are independent and time invariant. | tbd | To pass 7.7A.3 each component in the test vector must pass |
| 7.7F Spurious response for category NB1 | yes: it is assumed that the CW interferer causes errors, which are independent and time invariant. | tbd | To pass 7.7F each component in the test vector must pass |
| 7.8.1 Wide band Intermodulation | Unclear: errors are dependent on the data content of the interferer. Statistical independence is assumed. | tbd | To pass 7.8.1 each component in the test vector must pass |
| 7.8.1A.3 Wide-band intermodulation for CA (inter-band DL CA without UL CA) | Unclear: errors are dependent on the data content of the interferer. Statistical independence is assumed. | tbd | To pass 7.8.1A.3 each component in the test vector must pass |
| 7.8.1F Wide band Intermodulation for category NB1 | Unclear: errors are dependent on the data content of the interferer. Statistical independence is assumed. | tbd | To pass 7.8.1F each component in the test vector must pass |

# G.2A	Statistical testing of receiver characteristics with CA

## G.2A.1	General

G.2.1 applies.

## G.2A.2	Mapping throughput to error ratio

The test is defined by the following design principles (see clause G.x, Theory….):

1.	The standard concept is applied. (not the early decision concept).

2.	A second limit is introduced, defining the Bad DUT.

3.	To decide the test pass:

-	Supplier risk is applied based on the Bad DUT quality.

-	To decide the test fail.

-	Customer Risk is applied based on the specified DUT quality.

The test is defined by the following parameters:

1)	Limit Error Ratio = 0.05 (95% throughput is tested).

2)	Bad DUT factor M=1.5 (selectivity).

3)	Confidence level CL = 95% (for specified DUT and Bad DUT-quality).

## G.2A.3

## G.2A.4	Pass fail limits

Apply 1003 samples to the DUT per CC.

Decide pass per CC in case of ≤ 62 errors, otherwise fail.

NOTE 1:	The pass fail decision is done individually for each CC. The pass fail decision for one component in the test vector is as follows: pass if all CCs or SCC only according to the test cases pass, otherwise fail. The overall pass fail decision is according to clause G.2A.6

NOTE 2:	It is allowed to apply more samples to the DUT, common for all CCs, (e.g. up to an integer number of frames). Use the ratio (62/1003) for the pass fail decision.

NOTE 3:	62/1003 = 0.0618, the same test limit is used at the end of Table G.2.4-1

## G.2A.5	void

## G.2A.6	Test conditions for receiver tests with CA

Table G.2A.6-1: Test conditions for receiver tests with CA

| Test | Statistical independence | Number of components in the test vector, as specified in the test requirements and initial conditions of the applicable test Note 1 | Over all Pass/Fail condition Note 1 |
| --- | --- | --- | --- |
| 7.3A Reference sensitivity level for CA | Yes: the inherent receiver noise is assumed to be AWGN | tbd | To pass 7.3A each component in the test vector must pass |
| 7.4A Maximum input level for CA | Unclear: in case, clipping causes errors, errors are data dependent. Statistical independence is assumed. | tbd | To pass 7.4A each component in the test vector must pass |
| 7.5A Adjacent Channel Selectivity (ACS) for CA | Unclear: errors are data dependent on the interferers data. Statistical independence is assumed. | tbd | To pass 7.5A each component in the test vector must pass |
| 7.6.1A In-band blocking for CA | Unclear: errors are data dependent on the interferers data. Statistical independence is assumed. | tbd | To pass 7.6.1A each component in the test vector must pass |
| 7.6.2.A Out of-band blocking for CA | yes: it is assumed that the CW interferer causes errors, which are independent and time invariant. | tbd | To pass 7.6.2A, all except [tbd] components in the test vector must pass |
| 7.6.3A Narrow band blocking for CA | yes: it is assumed that the CW interferer causes errors, which are independent and time invariant. | tbd | To pass 7.6.3A each component in the test vector must pass |
| 7.7A Spurious response for CA | yes: it is assumed that the CW interferer causes errors, which are independent and time invariant. | tbd | To pass 7.7A each component in the test vector must pass |
| 7.8.1A Wide band Intermodulation | [place holder] |  |  |
| Note 1: A DUT with marginal performance on one component in the test vector passes this component with a confidence level of 95%, which is a false fail probability of 5%. All components in the test vector shall pass, to pass the test, For more than 1 component, performing marginal, there is an increased probability of a false fail for the test. |  |  |  |

# G.3	Statistical testing of Performance Requirements with throughput

## G.3.1	General

The test of receiver performance characteristics is two fold.

1.	A signal or a combination of signals is offered to the RX port(s) of the receiver.

2.	The ability of the receiver to demodulate /decode this signal is verified by measuring the throughput.

In (2) is the statistical aspect of the test and is treated here.

The minimum requirement for all receiver performance tests is either 70% or 30% of the maximum throughput.

All receiver performance tests are performed in fading conditions. In addition to the statistical considerations, this requires the definition of a minimum test time.

## G.3.2	Mapping throughput to error ratio

G.2.2 applies

## G.3.3	Design of the test

The test is defined by the following design principles (see clause G.x, Theory….):

1.	The standard concept is applied. (not the early decision concept)

2.	A second limit is introduced: The second limit is different, whether 30% or 70% throughput is tested.

3.	To decide the test pass:

Supplier risk is applied based on the Bad DUT quality

To decide the test fail:

Customer Risk is applied based on the specified DUT quality

The test is defined by the following parameters:

1a) Limit Error Ratio = 0.3 (in case 70% Throughput is tested) or

1b) Limit Throughput = 0.3 (in case 30% Throughput is tested)

2a) Bad DUT factor M=1.378 (selectivity)

2b) Bad DUT factor m=0.692 (selectivity)

justification see: TS 34.121 Clause F.6.3.3

3) Confidence level CL = 95% (for specified DUT and Bad DUT-quality)

## G.3.4	Pass Fail limit

Testing Throughput = 30%, then the test limit is

Number of successes (ACK) / number of samples ≥ 59 / 233

Testing Throughput = 70% then the test limit is

Number of fails (NACK and statDTX) / number of samples ≤ 66 / 184

We have to distinguish 3 cases:

a)	The duration for the number of samples (233 or 184) is greater than the minimum test time:

Then the number of samples (233 or 184) is predefined and the decision is done according to the number of events (59 successes or 66 fails)

b)	Since subframe 0 and 5 contain less bits than the remaining subframes, it is allowed to predefine a number of samples contained in an integer number of frames. In this case test-limit-ratio applies.

c)	The minimum test time is greater than the duration for the number of samples:

The minimum test time is predefined and the decision is done comparing the measured ratio at that instant against the test-limit-ratio.

NOTE:	The test time for most of the tests is governed by the Minimum Test Time

## G.3.5	Minimum Test time

If a pass fail decision in G.3.4 can be achieved earlier than the minimum test time, then the test shall not be decided, but continued until the minimum test time is elapsed.

The tables below contain the minimum number of subframes for FDD and TDD.

By simulations the minimum number of active subframes (carrying DL payload) was derived (MNAS),

then adding inactive subframes to the active ones (e.g. subframe 5 contains no DL payload. For TDD additional subframes contain no DL payload)

then rounding up to full thousand and

then adding a bias of 1000 (BMNSF).

Simulation method to derive minimum test time:

With a level, corresponding a throughput at the test limit (here 30% or 70% of the max. throughput) the preliminary throughput versus time converges towards the final throughput. The allowance of ± 0.2 dB around the above mentioned level is predefined by RAN5 to find the minimum test time. The allowance of ±0.2 dB maps through the function “final throughput versus level” into a throughput corridor. The minimum test time is achieved when the preliminary throughput escapes the corridor the last time. The two functions “final throughput versus level” and “preliminary throughput versus time” are simulation results, which are done individual for each demodulation scenario. HST-scenarios and scenarios with MNAS ≥ 50000 are derived differently.

Figure G.3.5-1: Simulation method to derive minimum test time

Table G.3.5-1: Minimum Test time for PDSCH Single Antenna Port Performance

| Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes (MNAS) to reach the corridor (Simulation, info only) | Minimum Number of Subframes (MNS) to reach the corridor (MNS = active and inactive subframes) (Calculation, info only) |  | Biased Minimum Number of SubFrames (BMNSF) BMNSF= +1000 (mandatory) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD | FDD | TDD |
| 1 | R.2 (10 MHz, full, QPSK, 1/3) (1x2 Low) EVA,5 | 38 764 | 43 072 | 77 528 | 45 000 | 79 000 |
| 2 | R.2 (10 MHz, full, QPSK, 1/3) (1x2 Low) ETU,70 | 2 764 | 3 072 | 5 528 | 5 000 | 7 000 |
| 3 | R.2 (10 MHz, full, QPSK, 1/3) (1x2 Low) ETU,300 | 1 424 | 1 583 | 2 848 | 3 000 | 4 000 |
| 4 | R.2 (10 MHz , full, QPSK, 1/3) (1x2) HST | 28 800 | NA | NA | 28 800 | 57 600 |
| 5 | R.4 (1.4 MHz, full, QPSK, 1/3) (1x2 Low) EVA,5 | 44 354 | 49 283 | 147 847 | 51 000 | 149 000 |
| 6 | R.3 (10 MHz, full, 16QAM, ½) (1x2 Low) EVA,5 | 39 020 | 43 356 | 78 040 | 45 000 | 80 000 |
|  | R.3-1 (5 MHz, full, 16QAM, ½) (1x2 Low) EVA,5 | 39 020 | 43 356 | 78 040 | 45 000 | 80 000 |
| 6 Rel-9 | R.3-1 (5MHz, full, 16QAM, ½) (1x2 Low) EVA5 | 39 020 (MNAS is not simulated, but estimated based on similar scenario in Table G.3.5-1 Test6) | 43 356 | 78 040 | 45 000 | 80 000 |
| 7 | R.3 (10 MHz, full, 16QAM, ½) (1x2 Low) ETU,70 | 1 366 | 1 518 | 2 732 | 3 000 | 4 000 |
|  | R.3-1 (5 MHz, full, 16QAM, ½) (1x2 Low) ETU70 | 1 366 | 1 518 | 2 732 | 3 000 | 4 000 |
| 7 Rel-9 | R.3-1 (5MHz, full, 16QAM, ½) (1x2 Low) ETU70 | 1 366 (MNAS is not simulated, but estimated based on similar scenario in Table G.3.5-1 Test7) | 1 518 | 2 732 | 3 000 | 4 000 |
| 8 | R.3 (10 MHz, full, 16QAM, ½) (1x2 High) ETU,300 | 3 189 | 3 544 | 6 378 | 5 000 | 8 000 |
|  | R.3-1 (5MHz, full, 16QAM, ½) (1x2 High) ETU300 | 3 189 | 3 544 | 6 378 | 5 000 | 8 000 |
| 8 Rel-9 | R.3-1 (5MHz, full, 16QAM, ½) (1x2 High) ETU300 | 3 189 (MNAS is not simulated, but estimated based on similar scenario in Table G.3.5-1 Test8) | 3 544 | 6 378 | 5 000 | 8 000 |
| 9 | R.5 (3 MHz, full, 64QAM, ¾) (1x2 Low) EVA,5 | 50 000 | 55 556 | 100 000 | 57 000 | 101 000 |
| 10 | R.6 (5 MHz, full, 64QAM, 3/4) (1x2 Low) EVA,5 | 48 847 | 54 275 | 97 694 | 56 000 | 99 000 |
| 10 Rel-9 | R.6-1 (5MHz, partial, 64QAM, ¾) (1x2 Low) EVA5 | 48 847 (MNAS is not simulated, but estimated based on similar scenario in Table G.3.5-1 Test10) | 54 275 | 97 694 | 56 000 | 99 000 |
| 11 | R.7 (10 MHz, full, 64QAM, ¾) (1x2 Low) EVA,5 | 46 524 | 51 694 | 93 048 | 53 000 | 95 000 |
| 11 Rel-9 | R.7-1 (10MHz, partial, 64QAM, ¾) (1x2 Low) EVA5 | 46 524 (MNAS is not simulated, but estimated based on similar scenario in Table G.3.5-1 Test11) | 51 694 | 93 048 | 53 000 | 95 000 |
| 12 | R.7 (10 MHz, full, 64QAM, ¾) (1x2 Low) ETU,70 | 4 722 | 5 247 | 9 444 | 7 000 | 11 000 |
| 12 Rel-9 | R.7-1 (10MHz, partial, 64QAM, ¾) (1x2 Low) ETU70 | 4 722 (MNAS is not simulated, but estimated based on similar scenario in Table G.3.5-1 Test12) | 5 247 | 9 444 | 7 000 | 11 000 |
| 13 | R.7 (10 MHz, full, 64 QAM, 3/4) (1x2High) EVA,5 | 100 000 | 111 112 | 200 000 | 113 000 | 201 000 |
| 13 Rel-9 | R.7-1 (10MHz, partial, 64QAM, ¾) (1x2 High) EVA5 | 100 000 (MNAS is not simulated, but estimated based on similar scenario in Table G.3.5-1 Test13) | 111 112 | 200 000 | 113 000 | 201 000 |
| 14 | R.8 (15 MHz, full, 64QAM, ¾) (1x2 Low) EVA,5 | 48 434 | 53 816 | 96 868 | 55 000 | 98 000 |
| 14 Rel-9 | R.8-1 (15MHz, partial, 64QAM, ¾) (1x2 Low) EVA5 | 48 434 (MNAS is not simulated, but estimated based on similar scenario in Table G.3.5-1 Test14) | 53 816 | 96 868 | 55 000 | 98 000 |
| 15 | R.9 (20 MHz, full, 64QAM,3/4) (1x2 Low) EVA,5 | 100 000 | 111 112 | 200 000 | 113 000 | 201 000 |
| 15 Rel-9 | R.9-1 (20MHz, partial, 64QAM, ¾) (1x2 Low) EVA5 | 100 000 (MNAS is not simulated, but estimated based on similar scenario in Table G.3.5-1 Test15) | 111 112 | 200 000 | 113 000 | 201 000 |
| 15 Rel-9 | R.9-2 (20MHz, partial, 64QAM, ¾) (1x2 Low) EVA5 | 100 000 (MNAS is not simulated, but estimated based on similar scenario in Table G.3.5-1 Test15) | 111 112 | 200 000 | 113 000 | 201 000 |
| 16 | R.0 (3 MHz, 1PRB,16QAM, ½) (1x2 Low) ETU,70 | 5 710 | 6 345 | 11 420 | 8 000 | 13 000 |
| 17 | R.1 (10MHz,1PRB,16QAM, ½) (1x2 Low) ETU,70 | 9 234 | 10 260 | 18 468 | 12 000 | 20 000 |
| 18 | R.1 (20MHz,1PRB,16QAM, ½) (1x2 Low) ETU,70 | 13 373 | 14 859 | 26 746 | 16 000 | 28 000 |

Table G.3.5-2: Minimum Test time for PDSCH Single Antenna Port Performance with 1 PRB

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) |  | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD | FDD | TDD |
| 1 | R.29 (10MHz,1PRB,16QAM,½) (1x2 Low) ETU,70 [MBSFN] | 5 246 | 17 487 | 17 487 | 19 000 | 19 000 |

Table G.3.5-3: Minimum Test time for PDSCH Transmit diversity 2x2

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) |  | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD | FDD | TDD |
| 1 | R.11 (10MHz, full, 16QAM, ½) (2x2 Med) EVA,5 [SFBC, Space Frequency Block Code] | 50 000 | 55 556 | 100 000 | 57 000 | 101 000 |
|  | R.11-1 (5MHz, full, 16QAM, ½) (2x2 Med) EVA5 [SFBC, Space Frequency Block Code] | 50 000 | 55 556 | 100 000 | 57 000 | 101 000 |
| 1 Rel-9 | R.11-2 (5MHz, full, 16QAM, ½) (2x2 Med) EVA5 [SFBC] | 50 000 (MNAS is not simulated, but estimated based on similar scenario in Table G.3.5-3 Test1) | 55 556 | 100 000 | 57 000 | 101 000 |
| 2 | R.10 (10MHz, Full, QPSK, 1/3) (2x2) HST [SFBC] | 28 800 | NA | NA | 28 800 | 57 600 |

Table G.3.5-3a: Minimum Test time for PDSCH Transmit diversity 2x2 for eICIC

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.11-4 (10MHz, full, QPSK, ½) (2x2 Med) EVA,5 [SFBC, Space Frequency Block Code] | 50 000 (MNAS is not simulated, but estimated based on similar scenario in Table G.3.5-3 Test1) | 224 000 | 501 000 |

Table G.3.5-3b: Minimum Test time for PDSCH Transmit diversity 2x2 for feICIC

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.11-4 (10MHz, full, QPSK, ½) (2x2 Med) EVA,5 [SFBC, Space Frequency Block Code] | 50 000 MNAS is not simulated, but estimated based on similar scenario in Table G.3.5-3 Test1 | 224 000 | 501 000 |

Table G.3.5-4: Minimum Test time for PDSCH Transmit diversity 4x2

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) |  | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD | FDD | TDD |
| 1 | R.12 (1.4MHz, full, QPSK, 1/3) (4x2 med) EPA,5 [SFBC-FSTD, SFBC-Frequency Shifted Transmit Diversity] | 150 000 | 166 667 | 300 000 | 168 000 | 301 000 |
| 1 Rel-9 | R.13 (10 MHz, full, QPSK, 1/3) (4x2 Low) ETU70 [SFBC-FSTD] | 10 000 (MNAS is not simulated, but estimated based on similar scenarios in Table G.3.5-4 Test1) | 11 112 | 20 000 | 13 000 | 21 000 |

Table G.3.5-5: Minimum Test time for PDSCH Open Loop Spacial Multiplexing 2x2

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) |  | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD | FDD | TDD |
| 1 FDD | R.11 (10MHz, Full, 16QAM, ½) (2x2 Low) EVA70 [LD-CDD, Large Delay-Cyclic Delay Diversity] | 7 600 | 8 445 | - | 10 000 | - |
| 1 TDD | R.11-1 (10MHz, Full, 16QAM, ½) (2x2 Low) EVA70 [LD-CDD, Large Delay-Cyclic Delay Diversity] | 7 600 | - | 19 000 | - | 20 000 |
| 2 | R.11-2 (5MHz, Full, 16QAM, ½) (2x2 Low) EVA,70 [LD-CDD, Large Delay-Cyclic Delay Diversity] | 7 600 (MNAS is not simulated, just follow the similar 10MHz test scenario in Test 1) | 8 445 | - | 10 000 | - |

Table G.3.5-5a: Minimum Test time for PDSCH Open Loop Spacial Multiplexing 2x2 for eICIC

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.11 (10MHz, Full, 16QAM, ½) (2x2 Low) EVA,5 [LD-CDD, Large Delay-Cyclic Delay Diversity, Non-MBSFN ABS] | 7 600 (MNAS is not simulated, but estimated based on similar scenario in Table G.3.5-5 Test1) | 35 000 | 77 000 |
| 1 | R.11 (10MHz, Full, 16QAM, ½) (2x2 Low) EVA,5 [LD-CDD, Large Delay-Cyclic Delay Diversity, MBSFN ABS] | 7 600 (MNAS is not simulated, but estimated based on similar scenario in Table G.3.5-3 Test1) | 77 000 | 77 000 |

Table G.3.5-5b: Minimum Test time for PDSCH Open Loop Spacial Multiplexing 2x2 for feICIC

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.11 (10MHz, Full, 16QAM, ½) (2x2 Low) EVA,5 [LD-CDD, Large Delay-Cyclic Delay Diversity] | 7 600 MNAS is not simulated, but estimated based on similar scenario in Table G.3.5-5 Test1 | 35 000 | 77 000 |
| 2 | R.35 (10MHz, full, 64QAM, ½) (2x2 Low) EVA,5 [LD-CDD, Large Delay-Cyclic Delay Diversity] | 7 600 MNAS is not simulated, but estimated based on similar scenario in Table G.3.5-5 Test1 | 35 000 | 77 000 |

Table G.3.5-6: Minimum Test time for PDSCH Open Loop Spacial Multiplexing 4x2

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) |  | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD | FDD | TDD |
| 1 | R.14 (10MHz, full, 16 QAM, ½) (4x2 low) EVA,70 [LD-CDD] | 4 860 | 5 400 | 12 150 | 7 000 | 14 000 |

Table G.3.5-7: Minimum Test time for PDSCH Closed LoopSingle/Multilayer Spacial Multiplexing 2x2

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) |  | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD | FDD | TDD |
| 1 | R.10 (10MHz, Full, QPSK,1/3) (2x2 Low) EVA5 [SCW, Single CodeWord] | 49 140 | 54 600 | 98 280 | 56 000 | 100 000 |
| 1A | R.10-2 (5MHz, Full, QPSK,1/3) (2x2 Low) EVA5 [SCW, Single CodeWord] | 49 140 | 54 600 | 98 280 | 56 000 | 100 000 |
| 2 | R.10 (10MHz, Full, QPSK, 1/3) (2x2 High) EPA5 [SCW] | 50 000 | 55 556 | 100 000 | 57 000 | 101 000 |
| 3 FDD | R.11 (10MHz,full, 16QAM, ½) (2x2Low) EVA5 [MCW, Multiple Code Word] | 34 266 | 38 074 | - | 40 000 | - |
| 3 TDD | R.11-1 (10MHz,full, 16QAM, ½) (2x2Low) EVA5 [MCW, Multiple Code Word] | 34 266 | - | 85 665 | - | 87 000 |
| 3 Rel-9 | R.35 (10MHz, full, 64QAM, ½) (2x2 Low) EPA5 [MCW] | 48 000 (MNAS is not simulated, but estimated based on similar scenarios in Table G.3.5-12 Test5) | 53 333 | 120 000 | 55 000 | 121 000 |
| 4 FDD | R.11 (10MHz, full, 16QAM, ½) (2x2Low) ETU70 [MCW] | 2 736 | 3 040 | - | 5 000 | - |
| 4 TDD | R.11-1 (10MHz, full, 16QAM, ½) (2x2Low) ETU70 [MCW] | 2 736 | - | 6840 | - | 8000 |
| 4A | R.11-2 (5MHz, full, 16QAM, ½) (2x2Low) ETU70 [MCW] | 2 736 | 3 040 | - | 5 000 | - |

Table G.3.5-8: Minimum Test time for PDSCH Closed Loop Single/Multilayer Spacial Multiplexing 4x2

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) |  | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD | FDD | TDD |
| 1 | R.13 (10 MHz, Full, QPSK, 1/3) (4x2 Low) EVA,5 [SCW] | 26 528 | 29 476 | 53 056 | 31 000 | 55 000 |
| 2 | R.14 (10MHz, Full, 16QAM, ½) (4x2low) EVA5 [MCW] | 26 066 | 28 963 | 65 165 | 30 000 | 67 000 |
| 2 Rel-9 | R.36 (10MHz, full, 64QAM 1/2) (4x2 Low) EPA5 [MCW] | 30 000 (MNAS is not simulated, but estimated based on similar scenarios in Table G.3.5-4 Test1) | 33 333 | 75 000 | 35 000 | 76 000 |

Table G.3.5-9: Minimum Test time for PDSCH Single-layer Spatial Multiplexing on antenna port 5 (Release 8 and forward)

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) |  | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD | FDD | TDD |
| 1 | R.25 (10 MHz, full, QPSK, 1/3) (1x2 Low) EPA,5 | 38 879 | 43 199 | 77 758 | 45 000 | 79 000 |
| 2 | R.26 (10MHz, full, 16QAM, ½) (1x2 Low) EPA5 | 47 781 | 53 090 | 95 562 | 55 000 | 97 000 |
| 3 | R.27 (10MHz, full, 64QAM, 3/4) (1x2 Low) EPA,5 | 48 685 | 54 095 | 97 370 | 56 000 | 99 000 |
| 4 | R.28 (10MHz, 1PRB, 16QAM, ½) (1x2 Low) EPA,5 | 100 000 | 111 112 | 200 000 | 113 000 | 201 000 |

Table G.3.5-10: Minimum Test time for PDSCH Single-layer Spatial Multiplexing on antenna port 5 (Release 9 and forward)

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 NA |  |  |  |  |
| 2 | R.26-1 (5MHz, full, 16QAM ½) (2x2 Low) EPA5 | Note: MNAS is not simulated. Because of same demodulation scenario except for antenna configuration, MNSF is reused from Table G.3.5-9, Test 2 | 55 000 | 97 000 |
| 3 | R.27-1 (10MHz, part, 64QAM 3/4) (2x2 Low) EPA,5 | Note: MNAS is not simulated. Because of same demodulation scenario except for antenna configuration, MNSF is reused from Table G.3.5-9, Test 3 | 56 000 | 99 000 |
| 4 NA |  |  |  |  |

Table G.3.5-11: Minimum Test time for PDSCH Single-layer Spatial Multiplexing on antenna port 7 or 8 without simultaneous transmission

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.31 (10 MHz, full, QPSK, 1/3) (2x2 Low) EVA,5 | Note: MNAS is not simulated. Because of same demodulation scenario except for RMC, antenna configuration, MNSF is reused from Table G.3.5-9, Test 1 | 45 000 | 79 000 |
| 2 | R.32 (10MHz, full, 16QAM, ½) (2x2 Medium) EPA5 | Note: MNAS is not simulated. Because of same demodulation scenario except for RMC, antenna configuration, MNSF is reused from Table G.3.5-9, Test 2 | 55 000 | 97 000 |
|  | R.32-1 (5MHz, full, 16QAM, ½) (2x2 Medium) EPA5 | Note: MNAS is not simulated. Because of same demodulation scenario except for RMC, channel BW, antenna configuration, MNSF is reused from Table G.3.5-9, Test 2 |  |  |
| 3 | R.33 (10MHz, full, 64QAM, 3/4) (2x2 Low) EPA,5 | Note: MNAS is not simulated. Because of same demodulation scenario except for RMC, antenna configuration, MNSF is reused from Table G.3.5-9, Test 3 | 56 000 | 99 000 |
|  | R.33-1 (10MHz, part, 64QAM, 3/4) (2x2 Low) EPA5 | Note: MNAS is not simulated. Because of same demodulation scenario except for RMC, allocation, antenna configuration, MNSF is reused from Table G.3.5-9, Test 3 |  |  |

Table G.3.5-12: Minimum Test time for PDSCH Single-layer Spatial Multiplexing on antenna port 7 or 8 with a simultaneous transmission

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 4 | R.32 (10MHz, full, 16QAM, 1/2) (2x2 Medium) EPA,5 | Note: MNAS is not simulated. Because of same demodulation scenario except for RMC number, antenna configuration, MNSF is reused from Table G.3.5-9, Test 2 | 55 000 | 97 000 |
| 5 | R.34 (10MHz, full, 64QAM, 1/2) (2x2 Low) EPA,5 | Note: MNAS is not simulated. Because of same demodulation scenario except for RMC number, target coding rate, antenna configuration, MNSF is reused from Table G.3.5-9, Test 3 | 56 000 | 99 000 |

Table G.3.5-12a: Minimum Test time for PDSCH Single-layer Spatial Multiplexing on antenna port 7 or 8 without simultaneous transmission for eDL-MIMO

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.43 FDD, R50 TDD (10 MHz, full, QSPK, 1/3) (2x2 Low) EVA,5 | Note: MNAS is not simulated. Because of same demodulation scenario except for RMC, antenna configuration, propagation condition, MNSF is reused from Table G.3.5-9, Test 1 | 45 000 | 79 000 |

Table G.3.5-12b: Minimum Test time for PDSCH Single-layer Spatial Multiplexing on antenna port 7 or 8 with a simultaneous transmission for eDL-MIMO

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 2 | R.50 FDD, R.44 TDD (10MHz, full, 64QAM, 1/2) (2x2 Low) EPA,5 | Note: MNAS is not simulated. Because of same demodulation scenario except for RMC, target coding rate, antenna configuration, MNSF is reused from Table G.3.5-9, Test 3 | 56 000 | 99 000 |

Table G.3.5-12c: Minimum Test time for PDSCH Single-layer Spatial Multiplexing for FeICIC

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.11 FDD, R11 TDD (10 MHz, full, 16QAM, 1/2) (2x2 High) EPA, 5 | Note: MNAS is not simulated. Because of same demodulation scenario except for RMC, antenna configuration, propagation condition, MNSF is reused from Table G.3.5-9, Test 1 | 55 000 | 97 000 |

Table G.3.5-13: Minimum Test time for PDSCH Dual-layer Spatial Multiplexing

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.31 (10 MHz, full, QPSK, 1/3) (2x2 Low) EVA,5 | Note: MNAS is not simulated. Because of same demodulation scenario except for RMC, antenna configuration, MNSF is reused from Table G.3.5-9, Test 1 | [45 000] | [79 000] |
| 2 | R.32 (10MHz, full, 16QAM, ½) (2x2 Medium) EPA5 | Note: MNAS is not simulated. Because of same demodulation scenario except for RMC, antenna configuration, MNSF is reused from Table G.3.5-9, Test 2 | [55 000] | [97 000] |

Table G.3.5-13a: Minimum Test time for PDSCH Dual-layer Spatial Multiplexing for eDL-MIMO

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.51 (10MHz, full, 16QAM, 1/2) (2x2 Low) EPA5 | Note: MNAS is not simulated. Because of same demodulation scenario except for RMC, antenna configuration, MNSF is reused from Table G.3.5-9, Test 2 | 55 000 | 97 000 |

Table G.3.5-14: Minimum Test time for PDSCH transmit Diversity 2x2 with TM3 Interference Model – Enhanced Performance Type A

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.46 (10MHz, full, QPSK, 1/3) (2x2 Low) EVA70 | Note: MNAS is not simulated. It is estimated based on other similar test cases using EVA70 or ETU70 propagation conditions | 15 000 | 27 000 |

Table G.3.5-15: Minimum Test time for PDSCH Closed Loop Single Layer Spatial Multiplexing 2x2 with TM4 Interference Model – Enhanced Performance Type A

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.47 (10MHz, full, 16QAM, 1/3) (2x2 Low) EVA5 | Note: MNAS is not simulated. It is estimated based on other similar test cases using EVA5 or ETU70 propagation conditions | 56 000 | 100 000 |

Table G.3.5-16: Minimum Test time for PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 with TM9 Interference Model – Enhanced Performance Type A

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.48 (10MHz, full, QPSK, 1/2) (4x2 Low) EVA5 | Note: MNAS is not simulated. It is estimated based on other similar test cases using EVA5 propagation conditions | 57 000 | 101 000 |

Table G.3.5-17: Minimum Test time for PDSCH Closed Loop Single layer Spacial Multiplexing 2x1 for UE Category 0

| Clause 8.9.1.1.2/8.9.1.2.2 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) |  | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | HD-FDD | TDD | HD-FDD | TDD |
| 1 | R.63 (10MHz, 1PRB, 64QAM, ½) (2x1 Low) EPA5 | 48 000 (MNAS is not simulated, but estimated based on similar scenarios in Table G.3.5-7 Test3 Rel-9) with HD-FDD pattern | 142 222 | 120 000 | 144 000 | 121 000 |

Table G.3.5-17a: Minimum Test time for PDSCH Closed Loop Single layer Spacial Multiplexing 2x1 for UE Category M1

| Clause 8.11.1.1.1/8.11.1.2.1 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) |  | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | HD-FDD | TDD | HD-FDD | TDD |
| 1 | R.79] (10MHz, 1PRB, 16QAM, ½) (2x1 Low) EPA5 | 48 000 (MNAS is not simulated, but estimated based on similar scenarios in Table G.3.5-7 Test3 Rel-9) with HD-FDD pattern | 240 000 | 120 000 | 241 000 | 121 000 |

Table G.3.5-18: Minimum Test time for PDSCH Open Loop Spatial Multiplexing 2x2– Enhanced Performance Type C

| Clause 8.2.1.3.1B/8.2.2.3.1B Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.11 FDD R.11-1 TDD (10MHz, Full, 16QAM, ½) (2x2 Medium) EVA70 [LD-CDD, Large Delay-Cyclic Delay Diversity] | 7 600 (MNAS is not simulated, just follow the similar 10MHz test scenario in Table G.3.5-5 Test 1 FDD) | 10 000 | 20 000 |

Table G.3.5-19: Minimum Test time for PDSCH Closed Loop Spatial Multiplexing 2x2– Enhanced Performance Type C

| Clause 8.2.1.4.2A/8.2.2.4.2A Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.11 FDD R.11-1 TDD (10MHz, full, 16QAM, ½) (2x2 Medium) ETU70 [MCW] | 2 736 (MNAS is not simulated, but estimated based on similar scenarios in Table G.3.5-7 Test4 FDD) | 5 000 | 8 000 |

Table G.3.5-20: Minimum Test time for PDSCH Open Loop Single/Multilayer Spatial Multiplexing 2x2 with TM1 Interference Model – Enhanced Performance Type C

| Clause 8.2.1.3.1C/8.2.2.3.1C Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.11-8 FDD R.11-10 TDD (10MHz, full, QPSK, 3/5, 1/2) (2x2 Medium) EVA70 | Note: MNAS is not simulated. It is estimated based on other similar test cases using EVA70 or ETU70 propagation conditions | 15 000 | 26 000 |

Table G.3.5-21: Minimum Test time for for CDM-multiplexed DM RS – Enhanced Performance Type C

| Clause 8.3.1.2.2/8.3.2.2.2 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.51 FDD R.32 TDD (10MHz, full, 16QAM, 1/2) (2x2 Medium) EPA5 | Note: MNAS is not simulated. Because of same demodulation scenario except for RMC, antenna configuration, MNSF is reused from Table G.3.5-9, Test 2 | [55 000] | [97 000] |

Table G.3.5-22: Minimum Test time for PDSCH transmit Diversity 2x2 with TM2 Interference Model – Enhanced Performance Type B

| Clause 8.2.1.2.5 / 8.2.2.2.6 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.11-10 FDD R.11-12 TDD (10MHz, full, QPSK, 0.67 FDD, 0.54-0.66 TDD) (2x2 Low) EPA5 | 48000 Note: MNAS is not simulated. It is estimated based on other similar test cases using EPA5 propagation conditions | 60 000 | 120 000 |

Table G.3.5-23: Minimum Test time for PDSCH transmit Diversity 2x2 with TM9 Interference Model – Enhanced Performance Type B

| Clause 8.2.1.2.6 / 8.2.2.2.7 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.11-9 FDD R.11-11 TDD (10MHz, full, QPSK, 0.58 FDD, 0.48-0.58 TDD) (2x2 Low) EPA5 | 48000 Note: MNAS is not simulated. It is estimated based on other similar test cases using EPA5 propagation conditions | 60 000 | 120 000 |

Table G.3.5-24: Minimum Test time for PDSCH Performance with DCI format 2D, non Quasi Co-located Antenna Ports, Same Cell ID and single NZP CSI-RS resource for CoMP

| Clause 8.3.1.3.1_F/ 8.3.2.4.1_F Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.52 FDD R.52 TDD (10MHz, full, 64QAM, 1/2) (2x2 Low) EPA,5 | Note: MNAS is not simulated. Because of same demodulation scenario except for RMC number, target coding rate, antenna configuration, MNSF is reused from Table G.3.5-9, Test 3 | 56 000 | 99 000 |
| 2 | R.52 FDD R.52 TDD (10MHz, full, 64QAM, 1/2) (2x2 Low) EPA,5 | Note: MNAS is not simulated. Because of same demodulation scenario except for RMC number, target coding rate, antenna configuration, MNSF is reused from Table G.3.5-9, Test 3 | 56 000 | 99 000 |

Table G.3.5-25: Minimum Test time for PDSCH Performance with DCI format 2D, non Quasi Co-located Antenna Ports, Same Cell ID and multiple NZP CSI-RS resources for CoMP

| Clause 8.3.1.3.2_F/ 8.3.2.4.2_F Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.53 FDD R.53 TDD (10MHz, full, 64QAM, 1/2) (2x2 Low) EPA,5 | Note: MNAS is not simulated. Because of same demodulation scenario except for RMC number, target coding rate, antenna configuration, MNSF is reused from Table G.3.5-9, Test 3 | 56 000 | 99 000 |
| 2 | R.53 FDD R.53 TDD (10MHz, full, 64QAM, 1/2) (2x2 Low) EPA,5 | Note: MNAS is not simulated. Because of same demodulation scenario except for RMC number, target coding rate, antenna configuration, MNSF is reused from Table G.3.5-9, Test 3 | 56 000 | 99 000 |

Table G.3.5-26: Minimum Test time for PDSCH Performance with DCI format 2D, non Quasi Co-located Antenna Ports, Different Cell ID, Colliding CRS and single NZP CSI-RS resource for CoMP

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.54 FDD R.54 TDD (10MHz, full, 16QAM, 1/2) (2x2 Low) EPA,5 | Note: MNAS is not simulated. Because of same demodulation scenario except for RMC number, antenna configuration, MNSF is reused from Table G.3.5-9, Test 2 | 55 000 | 97 000 |

Table G.3.5-27: Minimum Test time for PDSCH Closed Loop Single Layer Spatial Multiplexing 2x2 with TM4 Interference Model – Enhanced Performance Type B

| Clause 8.2.1.4.4 / 8.2.2.4.5 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.11-10 FDD R.11-12 TDD (10MHz, full, QPSK, 0.67 FDD, 0.54-0.66 TDD) (2x2 Low) EVA5 | 48000 Note: MNAS is not simulated. It is estimated based on other similar test cases using EVA5 propagation conditions | 60 000 | 120 000 |
| 2 | R.11-9 FDD R.11-11 TDD (10MHz, full, QPSK, 0.58 FDD, 0.48-0.58 TDD) (2x2 Low) EPA5 | 48000 Note: MNAS is not simulated. It is estimated based on other similar test cases using EPA5 propagation conditions | 60 000 | 120 000 |

Table G.3.5-28: Minimum Test time for PDSCH Transmit Diversity for UE Category 0

| Clause 8.9.1.1.1/8.9.1.2.1 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) |  | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | HD-FDD | TDD | HD-FDD | TDD |
| 1 | R.62 (10MHz, 16QAM, ½) (2x1 Low) EPA5 | 50 000 (MNAS is not simulated, but estimated based on similar scenarios in Table G.3.5-3 Test1) with HD-FDD pattern | 133 333 | 125 000 | 145 000 | 126 000 |

Table G.3.5-28a: Minimum Test time for PDSCH Transmit Diversity for UE Category M1

| Clause 8.11.1.1.3.1/8.11.1.1.3.1_1/ 8.11.1.2.3.1/8.11.1.2.3.1_1 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) |  | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | HD-FDD | TDD | HD-FDD | TDD |
| 1 | R.81 (10MHz, QPSK, 1/10) (2x1 Low) ETU1 | 194 560 (MNAS is not simulated but based on Table G.4.4-1) | 486 400 | 1 167 360 | 488 000 | 1 169 000 |
| 2 | R.79 (10MHz, 16QAM, ½) (2x1 Low) EPA5 | 48 000 (MNAS is not simulated, but estimated based on similar scenarios in Table G.3.5-9 Test2) with HD-FDD pattern | 240 000 | 120 000 | 241 000 | 121 000 |

Table G.3.5-28b: Minimum Test time for PDSCH Transmit Diversity for UE Category M2

| Clause 8.11.1.1.3.2/8.11.1.1.3.2_1/ 8.11.1.2.3.2/8.11.1.2.3.2_1 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) |  | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | HD-FDD | TDD | HD-FDD | TDD |
| 1 | R.90 (10MHz, QPSK, 1/3) (2x1 Low) EPA5 | (38 879 (MNAS is not simulated, but estimated based on similar scenarios in Table G.3.5-9 Test1) with HD-FDD pattern | 1 244 128 | 3 110 320 | 1 246 000 | 3 112 000 |
| 2 | R.91 (10MHz, QPSK, 1/10, 2x1 Low, ETU1) | (MNAS is not simulated but based on Table G.4.4-1) | 389 120 | 778 240 | 391 000 | 780 000 |

Table G.3.5-29: Minimum Test time for Closed Loop spatial multiplexing, 4 Layers spatial multiplexing with 4 Tx ports and 4 Rx ports

| Clause 8.10.1.1.8/ 8.10.1.2.8 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.74 (10MHz, full, 16QAM, 1/2) (4x4 Low) EPA5 | Note: MNAS is not simulated. It is estimated based on other similar test cases using EPA5 propagation conditions | 168 000 | 301 000 |

Table G.3.5-30: Minimum Test time for 4 Layer Spatial Multiplexing for 4 Rx (User-Specific Reference Symbols)

| Clause 8.10.1.1.9/ 8.10.1.2.9 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.75 (10MHz, full, 16QAM, 1/2) (4x4 Low) EPA5 | Note: MNAS is not simulated. It is estimated based on other similar test cases using EPA5 propagation conditions | 56 000 | 99 000 |

Table G.3.5-31: Minimum Test time for Open Loop spatial multiplexing, 3 Layers with 4 Tx ports and 4 Rx ports

| Clause 8.10.1.1.7/ 8.10.1.2.7 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.73 (10MHz, full, 64QAM, 0.43) (4x4 Low) EVA70 | 4 860 Note: MNAS is not simulated, but estimated based on the similar scenario in Table G.3.5-6 Test1 | 7 000 | 14 000 |

Table G.3.5-32: Minimum Test time for 4-Rx PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model – Enhanced Performance Type A

| Clause 8.10.1.1.3/8.10.1.2.3 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.47 (10MHz, full, 16QAM, 1/3) (2x4 Low) EVA5 | Note: MNAS is not simulated. It is estimated based on other similar test cases using EVA5 or ETU70 propagation conditions | 56 000 | 100 000 |

Table G.3.5A-33: Minimum Test time for PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category 0

| Clause 8.9.1.1.3/8.9.1.2.3 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) |  | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | HD-FDD | TDD | FDD | TDD |
| 1 | R.64 (10 MHz, 6l, QPSK, 1/3) (2x1 Low) EPA,5 | 38 879 (MNAS is not simulated. It is estimated based on other similar scenarios in Table G.3.5-9 Test 1) with HD-FDD pattern | 115 197 | 97 198 | 117 000 | 99 000 |

Table G.3.5-33a: Minimum Test time for PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category M1

| Clause 8.11.1.1.2/8.11.1.2.2 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) |  | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | HD-FDD | TDD | FDD | TDD |
| 1 | R.80 (10 MHz, 6l, QPSK, 1/3) (2x1 Low) EPA,5 | 38 879 (MNAS is not simulated. It is estimated based on other similar scenarios in Table G.3.5-9 Test 1) with HD-FDD pattern | 1 244 128 | 2 138 345 | 1 246 000 | 2 140 000 |

Table G.3.5-33b: Minimum Test time for PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category 1bis

| Clause 8.9.1.1.3_1/8.9.1.2.3_1 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) |  | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | HD-FDD | TDD | HD-FDD | TDD |
| 2 | R.86 (10MHz, 16QAM, 1/3) (2x1 Low) EPA5 | 50 000 (MNAS is not simulated, but estimated based on similar scenarios in Table G.3.5-3 Test1) with HD-FDD pattern | 133 333 | 125 000 | 145 000 | 126 000 |

Table G.3.5-34: Minimum Test time for 4-Rx PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 with TM9 Interference Model – Enhanced Performance Type A

| Clause 8.10.1.1.5 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.76 (10MHz, full, QPSK,) (2x4 Low) EVA5 | Note: MNAS is not simulated. It is estimated based on other similar test cases using EVA5 propagation conditions | 56 000 | 100 000 |

Table G.3.5-35: Minimum Test time for PDSCH Closed Loop Multi-layer Spatial Multiplexing 2x2 for 256QAM in DL

| Clause 8.2.1.4.1_H/ 8.2.2.4.1_H Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 3 | R.65 FDD (10MHz, full, 256QAM,) (2x2 Low) EVA5 R.65 TDD (20MHz, full, 256QAM,) (2x2 Low) EVA5 | 50 000 Note: MNAS is not simulated. It is estimated based on other similar test cases using EVA5 propagation conditions | 63 000 | 125 000 |

Table G.3.5-36: PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 without simultaneous transmission for eDL-MIMO for 256QAM in DL

| Clause 8.3.1.1.1_H /8.3.2.1.2_H Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 3 | R.66 FDD (10MHz, full, 256QAM,) (2x2 Low) EPA5 R.66 TDD (20MHz, full, 256QAM,) (2x2 Low) EPA5 | 50 000 Note: MNAS is not simulated. It is estimated based on other similar test cases using EPA5 propagation conditions | 63 000 | 125 000 |

Table G.3.5-37: PDSCH Transmit Diversity 2x4

| Clause 8.10.1.1.1 /8.10.1.2.1 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.11 (10MHz, full, 16QAM,) (2x4 Medium correlation A) EVA5 | 50 000 Note: MNAS is not simulated. It is estimated based on other similar test cases using EVA5 propagation conditions | 56 000 | 100 000 |

Table G.3.5-38: PDSCH Open Loop Spatial Multiplexing 2x4

| Clause 8.10.1.1.2 /8.10.1.2.2 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.11 (10MHz, full, 16QAM,) (2x4 Low) EVA70 | 7600 Note: MNAS is not simulated. It is estimated based on other similar test cases using EVA70 propagation conditions | 10 000 | 17 000 |

Table G.3.5-39: PDSCH Open Loop Spatial Multiplexing 2x4

| Clause 8.10.1.1.4 /8.10.1.2.4 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.36 (10MHz, full, 64QAM,) (4x4 Low) EPA5 | 50 000 Note: MNAS is not simulated. It is estimated based on other similar test cases using EPA5 propagation conditions | 56000 | 126 000 |
| 2 | R.72 (10MHz, full, 256QAM,) (4x4 Low) EPA5 | 50 000 Note: MNAS is not simulated. It is estimated based on other similar test cases using EPA5 propagation conditions | 56000 | 126 000 |

Table G.3.5-40: Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols)

| Clause 8.10.1.1.6 /8.10.1.2.6 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.51 (10MHz, full, 16QAM,) (2x4 Low) ETU5 | 50000 Note: MNAS is not simulated. It is estimated based on other similar test cases using EPA5 propagation conditions | 56 000 | 100 000 |

Table G.3.5-41: Minimum Test time for PDSCH Transmit Diversity for UE Category 1bis

| Clause 8.9.1.1.1_1/8.9.1.2.1_1 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) |  | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | HD-FDD | TDD | HD-FDD | TDD |
| 2 | R.84 (10MHz, 16QAM, ½) (2x1 Low) EPA5 | 50 000 (MNAS is not simulated, but estimated based on similar scenarios in Table G.3.5-3 Test1) with HD-FDD pattern | 133 333 | 125 000 | 145 000 | 126 000 |

Table G.3.5-42: Minimum Test time for PDSCH for UE Category NB1

| Clause 8.12.1.1.2 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | HD-FDD | TDD |
| 1 | R.NB.6 FDD (200kHz, QPSK,1/2) (1x1) EPA5 | 3609 | 3466000 | N/A |
| 2 | R.NB.6-1 FDD (200kHz, QPSK,1/2) (1x1) ETU1 | 3609 | 5544000 | N/A |

Table G.3.5-43: Minimum Test time for PDSCH for UE Category NB1

| Clause 8.12.1.1.1 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | HD-FDD | TDD |
| 1 | R.NB.5 FDD (200kHz, QPSK,1/3) (2x1 Low) EPA5 | 3040 | 99000 | N/A |
| 2 | R.NB.5 FDD (200kHz, QPSK,1/3) (2x1 Low) EPA5 | 97280 | 293000 |  |
| 3 | R.NB.5-1 FDD (200kHz, QPSK,1/3) (2x1 Low) ETU1 | 778240 | 1752000 | N/A |

Table G.3.5-44: Minimum Test time for PDSCH for UE Category NB2

| Clause 8.12.1.1.3 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | HD-FDD | TDD |
| 1 | R.NB.7 FDD (200kHz, QPSK,1/2) (1x1) EPA5 | 3609 | 44000 | N/A |

Table G.3.5-45: Minimum Test time for PDSCH Single-layer Spatial Multiplexing for FD-MIMO

| Clause 8.3.1.1.9/8.3.2.1.10 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.50 FDD, R.44 TDD (10MHz, full, 64QAM, 1/2) (2x2 Low) EPA5 | Note: MNAS is not simulated. Because of same demodulation scenario except for RMC, target coding rate, antenna configuration, MNSF is reused from Table G.3.5-9, Test 3 | 56 000 | 99 000 |

## G.3.6	Test conditions for receiver performance tests

Table G.3.6: Test conditions for receiver performance tests

Table G.3.6-1: Single Antenna Port Performance (Cell-specific Reference Symbols) for test case 8.2.1.1 and 8.2.2.1 demodulation of PDSCH

| Test | Statistical independence | Number of components in the test vector, as specified in the test requirements and initial conditions of the applicable test |  |  |  | Over all Pass/Fail condition |
| --- | --- | --- | --- | --- | --- | --- |
| 8.2.1.1 FDD PDSCH Single Antenna Port Performance (Cell-Specific Reference Symbols) | subframes are independent | CAT | 1 | 2 | 3-5 | To pass 8.2.1.1 and 8.2.2.1each component in the test vector must pass For UEs, supporting multiple E_UTRA-bands (number of bands =B), the number of repetitions must be multiplied by B. If a test is defined over a BW>(BW of the E_UTRA band), the test is not applicable and reduces the number of repetitions. If a test is defined over a BW, which is not supported in the E_UTRAN band, the test is not applicable and reduces the number of repetitions. |
|  |  | QPSK | 5 | 5 | 5 |  |
|  |  | 16QAM | 0 | 3 | 3 |  |
| 8.2.1.2 TDD PDSCH Single Antenna Port Performance (Cell-Specific Reference Symbols) | subframes are independent | 64 QAM | 1 | 6 | 7 |  |
|  |  | 1PRB | 4 | 4 | 4 |  |
|  |  | Σ | 10 | 18 | 19 |  |

Table G.3.6-2: Transmit Diversity Performance (Cell-specific Reference Symbols) for test case 8.2.1.2 and 8.2.2.2 demodulation of PDSCH

| Test | Statistical independence | Number of components in the test vector, as specified in the test requirements and initial conditions of the applicable test |  |  |  | Over all Pass/Fail condition |
| --- | --- | --- | --- | --- | --- | --- |
| 8.2.1.2 FDD PDSCH Transmit Diversity Performance (Cell-Specific Reference Symbols) | subframes are independent | CAT | 1 | 2 | 3-5 | To pass 8.2.1.2 and 8.2.2.2 each component in the test vector must pass For UEs, supporting multiple E_UTRA-bands (number of bands =B), the number of repetitions must be multiplied by B. If a test is defined over a BW, which is not supported in the E_UTRAN band, the test is not applicable and reduces the number of repetitions. |
|  |  | QPSK | 2 | 2 | 2 |  |
| 8.2.2.2 TDD PDSCH Transmit Diversity Performance (Cell-Specific Reference Symbols) | subframes are independent | 16QAM | 0 | 1 | 1 |  |
|  |  | Σ | 2 | 3 | 3 |  |

Table G.3.6-3: Open Loop Spatial Multiplexing Performance (Cell-specific Reference Symbols) for test case 8.2.1.3 and 8.2.2.3 demodulation of PDSCH

| Test | Statistical independence | Number of components in the test vector, as specified in the test requirements and initial conditions of the applicable test |  |  |  | Over all Pass/Fail condition |
| --- | --- | --- | --- | --- | --- | --- |
| 8.2.1.3 FDD PDSCH Open Loop Spatial Multiplexing Performance (Cell-Specific Reference Symbols) | subframes are independent | CAT | 1 | 2 | 3-5 | To pass 8.2.1.3 and 8.2.2.3 each component in the test vector must pass |
|  |  | 16QAM | 0 | 2 | 2 |  |
| 8.2.2.3 TDD PDSCH Open Loop Spatial Multiplexing Performance (Cell-Specific Reference Symbols) | subframes are independent | Σ | 0 | 2 | 2 |  |

Table G.3.6-4: Closed Loop Spatial Multiplexing Performance (Cell-specific Reference Symbols) for test case 8.2.1.4 and 8.2.2.4 demodulation of PDSCH

| Test | Statistical independence | Number of components in the test vector, as specified in the test requirements and initial conditions of the applicable test |  |  |  | Over all Pass/Fail condition |
| --- | --- | --- | --- | --- | --- | --- |
| 8.2.1.4 FDD PDSCH Closed Loop Spatial Multiplexing Performance (Cell-Specific Reference Symbols) | subframes are independent | CAT | 1 | 2 | 3-5 | To pass 8.2.1.4 and 8.2.2.4 each component in the test vector must pass |
|  |  | Single layer QPSK | 3 | 3 | 3 |  |
| 8.2.2.4 TDD PDSCH Closed Loop Spatial Multiplexing Performance (Cell-Specific Reference Symbols) | subframes are independent | Multi layer 16QAM | 0 | 3 | 3 |  |
|  |  | Σ | 3 | 6 | 6 |  |

Table G.3.6-5: TDD PDSCH Single-layer Spatial Multiplexing on antenna port 5 (Release 8 and forward)

| Test | Statistical independence | Number of components in the test vector, as specified in the test requirements and initial conditions of the applicable test |  |  |  | Over all Pass/Fail condition |
| --- | --- | --- | --- | --- | --- | --- |
| 8.3.2.1.1 TDD PDSCH Single-layer Spatial Multiplexing on antenna port 5 (Release 8 and forward) | subframes are independent | CAT | 1 | 2 | 3-5 | To pass 8.3.2.1 each component in the test vector must pass |
|  |  | QPSK | 1 | 1 | 1 |  |
|  |  | 16QAM | 1 | 2 | 2 |  |
|  |  | 64 QAM | 0 | 1 | 1 |  |
|  |  | Σ | 2 | 4 | 4 |  |

Table G.3.6-6: TDD PDSCH Single-layer Spatial Multiplexing on antenna port 5 (Release 9 and forward)

| Test | Statistical independence | Number of components in the test vector, as specified in the test requirements and initial conditions of the applicable test |  |  |  | Over all Pass/Fail condition |
| --- | --- | --- | --- | --- | --- | --- |
| 8.3.2.1.1_1 TDD PDSCH Single-layer Spatial Multiplexing on antenna port 5 (Release 9 and forward) | subframes are independent | CAT | 1 | 2 | 3-5 | To pass 8.3.2.1.1_1 each component in the test vector must pass |
|  |  | 16QAM | 1 | 0 | 0 |  |
|  |  | 64 QAM | 1 | 0 | 0 |  |
|  |  | Σ | 2 | 0 | 0 |  |

Table G.3.6-7: TDD PDSCH Single-layer Spatial Multiplexing on antenna port 7 or 8 without a simultaneous transmission

| Test | Statistical independence | Number of components in the test vector, as specified in the test requirements and initial conditions of the applicable test |  |  |  | Over all Pass/Fail condition |
| --- | --- | --- | --- | --- | --- | --- |
| 8.3.2.1.2 TDD PDSCH Single-layer Spatial Multiplexing on antenna port 7 or 8 without a simultaneous transmission | subframes are independent | CAT | 1 | 2-5 |  | To pass 8.3.2.1.2 each component in the test vector must pass |
|  |  | QPSK | 1 | 1 |  |  |
|  |  | 16QAM | 1 | 1 |  |  |
|  |  | 64 QAM | 1 | 1 |  |  |
|  |  | Σ | 3 | 3 |  |  |

Table G.3.6-8: TDD PDSCH Single-layer Spatial Multiplexing on antenna port 7 or 8 with a simultaneous transmission

| Test | Statistical independence | Number of components in the test vector, as specified in the test requirements and initial conditions of the applicable test |  |  |  | Over all Pass/Fail condition |
| --- | --- | --- | --- | --- | --- | --- |
| 8.3.2.1.3 TDD PDSCH Single-layer Spatial Multiplexing on antenna port 7 or 8 with a simultaneous transmission | subframes are independent | CAT | 1 | 2-5 |  | To pass 8.3.2.1.3 each component in the test vector must pass |
|  |  | 16QAM | 0 | 1 |  |  |
|  |  | 64 QAM | 0 | 1 |  |  |
|  |  | Σ | 0 | 2 |  |  |

Table G.3.6-8a: PDSCH Single-layer Spatial Multiplexing on antenna port 7 or 8 without a simultaneous transmission for eDL-MIMO

| Test | Statistical independence | Number of components in the test vector, as specified in the test requirements and initial conditions of the applicable test |  |  |  |  | Over all Pass/Fail condition |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.3.1.1.1_D FDD PDSCH Single-layer Spatial Multiplexing on antenna port 7 or 8 without a simultaneous transmission | subframes are independent | CAT | 1-8 |  |  | To pass 8.3.1.1.1_D and 8.3.2.1.2_D each component in the test vector must pass |  |
|  |  | QPSK | 1 |  |  |  |  |
| 8.3.2.1.2_D TDD PDSCH Single-layer Spatial Multiplexing on antenna port 7 or 8 without a simultaneous transmission | subframes are independent | QPSK | 1 |  |  |  |  |
|  |  | Σ | 2 |  |  |  |  |

Table G.3.6-8b: PDSCH Single-layer Spatial Multiplexing on antenna port 7 or 8 with a simultaneous transmission for eDL-MIMO

| Test | Statistical independence | Number of components in the test vector, as specified in the test requirements and initial conditions of the applicable test |  |  |  |  | Over all Pass/Fail condition |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.3.1.1.2_D FDD PDSCH Single-layer Spatial Multiplexing on antenna port 7 or 8 with a simultaneous transmission for eDL-MIMO | subframes are independent | CAT | 1 | 2-8 |  | To pass 8.3.1.1.2_D and 8.3.2.1.3_D each component in the test vector must pass |  |
|  |  | 64QAM | 0 | 1 |  |  |  |
| 8.3.2.1.3_D TDD PDSCH Single-layer Spatial Multiplexing on antenna port 7 or 8 with a simultaneous transmission for eDL-MIMO | subframes are independent | 64QAM | 0 | 1 |  |  |  |
|  |  | Σ | 0 | 2 |  |  |  |

Table G.3.6-8c: PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 with TM9 Interference Model - Enhanced Performance Requirement Type A

| Test | Statistical independence | Number of components in the test vector, as specified in the test requirements and initial conditions of the applicable test |  |  |  | Over all Pass/Fail condition |
| --- | --- | --- | --- | --- | --- | --- |
| 8.3.1.1.3 FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 with TM9 Interference Model - Enhanced Performance Requirement Type A | subframes are independent | CAT | 1-8 |  |  | To pass 8.3.1.1.3 and 8.3.2.1.4 each component in the test vector must pass |
|  |  | QPSK | 1 |  |  |  |
| 8.3.2.1.4 TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 with TM9 Interference Model - Enhanced Performance Requirement Type A | subframes are independent | QPSK | 1 |  |  |  |
|  |  | Σ | 2 |  |  |  |

Table G.3.6-9: TDD PDSCH Dual-layer Spatial Multiplexing

| Test | Statistical independence | Number of components in the test vector, as specified in the test requirements and initial conditions of the applicable test |  |  |  | Over all Pass/Fail condition |
| --- | --- | --- | --- | --- | --- | --- |
| 8.3.2.2.1 TDD PDSCH Dual-layer Spatial Multiplexing | subframes are independent | CAT | 1 | 2 | 3-5 | To pass 8.3.2.2.1 each component in the test vector must pass |
|  |  | QPSK | 1 | 1 | 1 |  |
|  |  | 16QAM | 1 | 2 | 2 |  |
|  |  | 64 QAM | 0 | 1 | 1 |  |
|  |  | Σ | 2 | 4 | 4 |  |

Table G.3.6-9a: PDSCH Dual-layer Spatial Multiplexing for eDL-MIMO

| Test |  | Statistical independence |  | Number of components in the test vector, as specified in the test requirements and initial conditions of the applicable test |  |  |  |  | Over all Pass/Fail condition |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.3.1.2.1_D FDD PDSCH Dual-layer Spatial Multiplexing for eDL-MIMO | subframes are independent |  | CAT |  | 1 | 2-8 |  | To pass 8.3.1.2.1_D and 8.3.2.2.1_D each component in the test vector must pass |  |
|  |  |  | 16QAM |  | 0 | 1 |  |  |  |
| 8.3.2.2.1_D TDD PDSCH Dual-layer Spatial Multiplexing for eDL-MIMO | subframes are independent |  | 16QAM |  | 0 | 1 |  |  |  |
|  |  |  | Σ |  | 0 | 2 |  |  |  |

# G.3A	Statistical testing of Performance Requirements with throughput for CA/DC/LAA

## G.3A.1	General

The minimum requirements for performance tests in fading conditions in clause 8 with respect to CA/DC are 70% of the maximum throughput. The minimum requirements in static conditions in clause 8 with respect to CA/DC are [TBD]. Statistical tests in static propagation conditions lead to a statistically justified number of samples (testtime) and a test limit. The fading conditions require a minimum test time, overriding the statistically justified test time. It can be found in Tables G.3A.5. The statistically justified test limit is also used for the tests under fading conditions. The throughput is measured on both carriers in parallel (unless otherwise stated). The test for both carriers need the same time. The sum of the CC’s throughput is compared against the limit, where the limit is the sum of the individual carrier’s limit.

## G.3A.2	Mapping throughput to error ratio

G.2.2 applies separate for each CC

## G.3A.3	Design of the test

The test is defined by the following design principles (see clause G.x, Theory….):

1.	The standard concept is applied (not the early decision concept).

2.	A second limit is introduced, defining the Bad DUT.

3.	To decide the test pass:

-	Supplier risk is applied based on the Bad DUT quality.

-	To decide the test fails.

-	Customer Risk is applied based on the specified DUT quality.

The test is defined by the following parameters:

1)	Limit Error Ratio = 0.3. 
 (in case 70% throughput is tested , otherwise [TBD]).

2)	Bad DUT factor M = 1.378 (selectivity) justification see: TS 34.121 Clause F.6.3.3. 
(M = 1.378 is tied to 70% throughput, otherwise M is [TBD].)

3)	Confidence level CL = 95% (for specified DUT and Bad DUT-quality).

## G.3A.4	Pass Fail limit

Testing with the parameters from G.3A.3 (70% throughput, M = 1.378, CL95%): Apply 184 samples to the DUT per CC and count the errors for each CC. The test limit to pass for one CC is ≤ 66 errors, however this is not individually applicable for CA/DC.

Pass fail decision for one test point in CA/DC: The sum of the CC’s errors is compared against the test limit, where the test limit is the sum of the individual carrier’s test limit.

It is allowed to apply more samples, in parallel for all CCs, to the DUT (e.g. up to an integer number of frames).

In fading conditions it is necessary to apply more samples, in parallel for all CCs, to the DUT, as in fading conditions the minimum test time overrides the statistically justified test time.

When more samples are applied, decide against the ratio 66/184 = 0.3587.

Testing with parameters from G.3A.3 ([TBD]% throughput, M = [TBD], CL = 95%) is [TBD].

## G.3A.5	Minimum test time

In contrast to G.3.5, where the minimum test time is derived from simulations, the test time here is selected without simulation utilising test time information from similar demodulation scenarios in non-CA test cases and while maintaining the reasonable testing time.

Note 	MNAS values in this clause may need to be changed in future if current MNAS values turn out to be too short for giving stable CA/DC throughput results.

Table G.3A.5-1: Minimum Test time FDD PDSCH Single Antenna Port Performance for CA (2DL CA)

| Clause 8.2.1.1.1_A.1 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD |
| 1 | R.2FDD (2x10 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 2 | R.42FDD (2x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 3 | R.42-3 FDD + R.42-2 FDD (15MHz+5MHz,full,QPSK,1/3) (1x2 Low) EVA5) | 50 000 | 55 556 |

Table G.3A.5-1A: Minimum Test time FDD PDSCH Single Antenna Port Performance for CA (3DL CA)

| Clause 8.2.1.1.1_A.2 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD |
| 1 | R.42FDD (3x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 2 | R.42FDD + R.42-3FDD (20+20+15 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 3 | R.42FDD + R.2FDD (20+20+10 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 4 | R.42FDD + R.42-3FDD (20+15+15 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 5 | R.42FDD + R.42-3FDD + R.2FDD (20+15+10 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 6 | R.42FDD + R.2FDD (20+10+10 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 7 | R.42-3FDD + R.2FDD (15+15+10 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 8 | R.42FDD + R.2FDD + R.42-2FDD (20+10+5 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 9 | R.42FDD + R.42-3FDD + R.42-2FDD (20+15+5 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |

Table G.3A.5-1B: Minimum Test time FDD PDSCH Single Antenna Port Performance for CA (4DL CA)

| Clause 8.2.1.1.1_A.4 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD |
| 1 | R.42FDD (4x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 2 | R.2FDD + R.42FDD (10+20+20+20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 3 | R.2FDD + R.42FDD (10+10+20+20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 4 | R.42-2FDD + R.2FDD + R.42FDD (5+10+20+20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 5 | R.42-2FDD + R.2FDD + R.42FDD (5+10+10+20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |

Table G.3A.5-1C: Minimum Test time FDD PDSCH Single Antenna Port Performance for CA (5DL CA)

| Clause 8.2.1.1.1_A.5 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD |
| 1 | R.42FDD (5x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 2 | R.42-3FDD + R.42FDD (15+4x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 3 | R.2FDD + R.42FDD (10+4x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 4 | R.2FDD + R.42FDD (2x10+3x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 5 | R.42-2FDD + R.2FDD + R.42FDD (5+10+3x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 6 | R.2FDD + R.42FDD (3x10+2x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 7 | R.2FDD + R.42FDD (4x10+20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |

Table G.3A.5-1D: Minimum Test time FDD PDSCH Single Antenna Port Performance for CA (6DL CA)

| Clause 8.2.1.1.1_A.6 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD |
| 1 | R.42FDD (6x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |

Table G.3A.5-1E: Minimum Test time FDD PDSCH Single Antenna Port Performance for CA (7DL CA)

| Clause 8.2.1.1.1_A.7 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD |
| 1 | R.42FDD (7x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |

Table G.3A.5-2: Minimum Test time FDD PDSCH Open Loop Spatial Multiplexing 2x2 (2DL CA)

| Clause 8.2.1.3.1_A.1 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD |
| 1 | R.11 FDD (2x10 MHz, full 16QAM, 1/2), (2x2 Low) EVA70 | 10 000 | 11 112 |
| 2 | R.30 FDD (2x20 MHz, full 16QAM, 1/2), (2x2 Low) EVA70 | 10 000 | 11 112 |
| 3 | R.11-2 FDD (2x5 MHz, full 16QAM, 1/2), (2x2 Low) EVA70 | 10 000 | 11 112 |
| 4 | FFS | FFS | FFS |
| 5 | R.11-7 FDD + R.11-2 FDD (15MHz+5MHz, full 16QAM, 1/2), (2x2 Low) EVA70 | 10 000 | 11 112 |

Table G.3A.5-2A: Minimum Test time FDD PDSCH Open Loop Spatial Multiplexing 2x2 (3DL CA)

| Clause 8.2.1.3.1_A.2 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD |
| 1 | R.30 FDD (3x20 MHz, full 16QAM, 1/2), (2x2 Low) EVA70 | 10 000 | 11 112 |
| 2 | R.30 FDD + R.11-7 FDD (20+20+15 MHz, full 16QAM, 1/2), (2x2 Low) EVA70 | 10 000 | 11 112 |
| 3 | R.30 FDD + R.11 FDD (20+20+10 MHz, full 16QAM, 1/2), (2x2 Low) EVA70 | 10 000 | 11 112 |
| 4 | R.30 FDD + R.11-7 FDD (20+15+15 MHz, full 16QAM, 1/2), (2x2 Low) EVA70 | 10 000 | 11 112 |
| 5 | R.30 FDD + R.11-7 FDD + R.11 FDD (20+15+10 MHz, full 16QAM, 1/2), (2x2 Low) EVA70 | 10 000 | 11 112 |
| 6 | R.30 FDD + R.11 FDD (20+10+10 MHz, full 16QAM, 1/2), (2x2 Low) EVA70 | 10 000 | 11 112 |
| 7 | R.11-7 FDD + R.11 FDD (15+15+10 MHz, full 16QAM, 1/2), (2x2 Low) EVA70 | 10 000 | 11 112 |
| 8 | R.30 FDD + R.11 FDD + R.11-2 FDD (20+10+5 MHz, full 16QAM, 1/2), (2x2 Low) EVA70 | 10 000 | 11 112 |
| 9 | R.30 FDD + R.11-7 FDD + R.11-2 FDD (20+15+5 MHz, full 16QAM, 1/2), (2x2 Low) EVA70 | 10 000 | 11 112 |

Table G.3A.5-2B: Minimum Test time FDD PDSCH Open Loop Spatial Multiplexing 2x2 (4DL CA)

| Clause 8.2.1.3.1_A.3 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD |
| 1 | R.30 FDD (4x20 MHz, full 16QAM, 1/2), (2x2 Low) EVA70 | 10 000 | 11 112 |
| 2 | R.11 FDD + R.30 FDD (10+20+20+20 MHz, full 16QAM, 1/2), (2x2 Low) EVA70 | 10 000 | 11 112 |
| 3 | R.11 FDD + R.30 FDD (10+10+20+20 MHz, full 16QAM, 1/2), (2x2 Low) EVA70 | 10 000 | 11 112 |
| 4 | R.11-2 FDD + R.11 FDD + R.30 FDD (5+10+20+20 MHz, full 16QAM, 1/2), (2x2 Low) EVA70 | 10 000 | 11 112 |
| 5 | R.11-2 FDD + R.11 FDD + R.30 FDD (5+10+10+20 MHz, full 16QAM, 1/2), (2x2 Low) EVA70 | 10 000 | 11 112 |

Table G.3A.5-2C: Minimum Test time FDD PDSCH Open Loop Spatial Multiplexing 2x2 (5DL CA)

| Clause 8.2.1.3.1_A.4 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD |
| 1 | R.30FDD (5x20 MHz,full,16QAM,1/2) (2x2 Low) EVA70 | 10 000 | 11 112 |
| 2 | R.11-7FDD + R.30FDD (15+4x20 MHz,full,16QAM,1/2) (2x2 Low) EVA70 | 10 000 | 11 112 |
| 3 | R.11FDD + R.30FDD (10+4x20 MHz,full,16QAM,1/2) (2x2 Low) EVA70 | 10 000 | 11 112 |
| 4 | R.11FDD + R.30FDD (2x10+3x20 MHz,full,16QAM,1/2) (2x2 Low) EVA70 | 10 000 | 11 112 |
| 5 | R.11-2FDD + R.11FDD + R.30FDD (5+10+3x20 MHz,full,16QAM,1/2) (2x2 Low) EVA70 | 10 000 | 11 112 |
| 6 | R.11FDD + R.30FDD (3x10+2x20 MHz,full,16QAM,1/2) (2x2 Low) EVA70 | 10 000 | 11 112 |
| 7 | R.11FDD + R.30FDD (4x10+20 MHz,full,16QAM,1/2) (2x2 Low) EVA70 | 10 000 | 11 112 |

Table G.3A.5-2D: Minimum Test time FDD PDSCH Open Loop Spatial Multiplexing 2x2 (6DL CA)

| Clause 8.2.1.3.1_A.5 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD |
| 1 | R.30FDD (6x20 MHz,full,16QAM,1/2) (2x2 Low) EVA70 | 10 000 | 11 112 |

Table G.3A.5-2E: Minimum Test time FDD PDSCH Open Loop Spatial Multiplexing 2x2 (7DL CA)

| Clause 8.2.1.3.1_A.6 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD |
| 1 | R.30FDD (7x20 MHz,full,16QAM,1/2) (2x2 Low) EVA70 | 10 000 | 11 112 |

Table G.3A.5-3: Minimum Test time FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 (2DL CA)

| Clause 8.2.1.4.2_A.1 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD |
| 1 | R.14-3 FDD (2x20 MHz, full,16QAM,1/2) (4x2 Low) EVA5 | 50 000 | 55 556 |
| 2 | R.14FDD (2x10 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 55 556 |
| 3 | R.14-7 FDD + R.14-6 FDD (15MHz+5MHz,full,16QAM,1/2) (4x2 Low) EVA5 | 50 000 | 55 556 |

Table G.3A.5-3A: Minimum Test time FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 (3DL CA)

| Clause 8.2.1.4.2_A.2 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD |
| 1 | R.14-3 FDD (3x20 MHz,full,16QAM,1/2) (4x2 Low) EVA5 | 50 000 | 55 556 |
| 2 | R.14-3 FDD + R.14-7 FDD (20+20+15 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 55 556 |
| 3 | R.14-3 FDD + R.14 FDD (20+20+10 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 55 556 |
| 4 | R.14-3 FDD + R.14-7 FDD (20+15+15 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 55 556 |
| 5 | R.14-3 FDD + R.14-7 FDD + R.14 FDD (20+15+10 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 55 556 |
| 6 | R.14-3 FDD + R.14 FDD (20+10+10 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 55 556 |
| 7 | R.14-7 FDD + R.14 FDD (15+15+10 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 55 556 |
| 8 | R.14-3 FDD + R.14 FDD + R.14-6 FDD (20+10+5 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 55 556 |
| 9 | R.14-3 FDD + R.14-7 FDD + R.14-6 FDD (20+15+5 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 55 556 |

Table G.3A.5-3B: Minimum Test time FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 (4DL CA)

| Clause 8.2.1.4.2_A.3 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD |
| 1 | R.14-3 FDD (4x20 MHz,full,16QAM,1/2) (4x2 Low) EVA5 | 50 000 | 55 556 |
| 2 | R.14-3 FDD + R.14-7 FDD (20+20+20+15 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 55 556 |
| 3 | R.14-3 FDD + R.14 FDD (20+20+20+10 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 55 556 |
| 4 | R.14-3 FDD + R.14-7 FDD (20+20+15+15 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 55 556 |
| 5 | R.14-3 FDD + R.14-7 FDD + R.14 FDD (20+20+15+10 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 55 556 |
| 6 | R.14-3 FDD + R.14 FDD (20+20+10+10 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 55 556 |
| 7 | R.14-3 FDD + R.14-7 FDD + R.14 FDD (20+15+15+10 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 55 556 |
| 8 | R.14-3 FDD + R.14-7 FDD + R.14 FDD (20+15+10+10 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 55 556 |
| 9 | R.14-3 FDD + R.14 FDD (20+10+10+10MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 55 556 |
| 10 | R.14-7 FDD + R.14 FDD (15+15+15+10 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 55 556 |
| 11 | R.14-3 FDD + R.14 FDD + R.14-6 FDD (20+20+10+5 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 55 556 |
| 12 | R.14-6FDD + R.14 FDD + R.14-3 FDD (5+10+10+10+20MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 55 556 |

Table G.3A.5-3C: Minimum Test time FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 (5DL CA)

| Clause 8.2.1.4.2_A.4 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD |
| 1 | R.14-3 FDD (5x20 MHz,full,16QAM,1/2) (4x2 Low) EVA5 | 50 000 | 55 556 |
| 2 | R.14-7 FDD + R.14-3 FDD (15 + 4x20MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 55 556 |
| 3 | R.14 FDD + R.14-3 FDD (10 + 4x20 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 55 556 |
| 4 | R.14 FDD + R.14-3 FDD (2x10 + 3x20 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 55 556 |
| 5 | R.14-6 FDD + R.14 FDD + R.14-3 FDD (5 + 10 + 3x20 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 55 556 |
| 6 | R.14 FDD + R.14-3 FDD (3x10 + 2x20 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 55 556 |
| 7 | R.14 FDD + R.14-3 FDD (4x10 + 20 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 55 556 |

Table G.3A.5-3D: Minimum Test time FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 (6DL CA)

| Clause 8.2.1.4.2_A.5 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD |
| 1 | R.14-3 FDD (6x20 MHz,full,16QAM,1/2) (4x2 Low) EVA5 | 50 000 | 55 556 |

Table G.3A.5-3E: Minimum Test time FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 (7DL CA)

| Clause 8.2.1.4.2_A.6 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD |
| 1 | R.14-3 FDD (7x20 MHz,full,16QAM,1/2) (4x2 Low) EVA5 | 50 000 | 55 556 |

Table G.3A.5-4: Minimum Test time FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for Dual Connectivity

| Clause 8.2.1.4.3A Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD |
| 1 | R.14-3FDD (2x20 MHz, full,16QAM,1/2) (4x2 Low) EVA5 | 50 000 | 55 556 |
| 2 | R.14-7FDD + R.14-3FDD (15+20 MHz,full,16QAM,1/2) (4x2 Low) EVA5 | 50 000 | 55 556 |
| 3 | R.14FDD + R.14-3FDD (10+20 MHz,full,16QAM,1/2) (4x2 Low) EVA5 | 50 000 | 55 556 |
| 4 | R.14-7FDD (2x15 MHz,full,16QAM,1/2) (4x2 Low) EVA5 | 50 000 | 55 556 |
| 5 | R.14FDD (2x10 MHz,full,16QAM,1/2) (4x2 Low) EVA5 | 50 000 | 55 556 |

Table G.3A.5-4A: Void

Table G.3A.5-5: Void

Table G.3A.5-6: Minimum Test time TDD PDSCH Single Antenna Port Performance for CA (2DL CA)

| Clause 8.2.2.1.1_A.1 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | TDD |
| 1 | R.42TDD (2x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 2 | R.42TDD (20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
|  | R.42-3 TDD (1x2 Low) EVA5 |  |  |

Table G.3A.5-6A: Minimum Test time TDD PDSCH Single Antenna Port Performance for CA (3DL CA)

| Clause 8.2.2.1.1_A.2 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD |
| 1 | R.42TDD (3x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 2 | R.42TDD + R.42-3TDD (20+20+15 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |

Table G.3A.5-6B: Minimum Test time TDD PDSCH Single Antenna Port Performance for CA (4DL CA)

| Clause 8.2.2.1.1_A.3 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD |
| 1 | R.42TDD (4x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 2 | R.42TDD + R.42-3TDD (20+20+20+15 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |

Table G.3A.5-6C: Minimum Test time TDD PDSCH Single Antenna Port Performance for CA (5DL CA)

| Clause 8.2.2.1.1_A.4 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD |
| 1 | R.42TDD (5x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 2 | R.42TDD + R.42-3TDD (20+20+20+20+15 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |

Table G.3A.5-7: Minimum Test time TDD PDSCH Open Loop Spatial Multiplexing 2x2 2DL CA

| Clause 8.2.2.3.1_A.1 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | TDD |
| 1 | R.30-1 TDD (2x20 MHz, full,16QAM,1/2) (2x2 Low) EVA70 | 10 000 | 20 000 |
| 2 | R.30-1 TDD (20 MHz, full,16QAM,1/2) (2x2 Low) EVA70 | 10 000 | 20 000 |
|  | [TBD] (2x2 Low) EVA70 |  |  |

Table G.3A.5-7A: Minimum Test time TDD PDSCH Open Loop Spatial Multiplexing 2x2 3DL CA

| Clause 8.2.2.3.1_A.2 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | TDD |
| 1 | R.30-1 TDD (3x20 MHz, full,16QAM,1/2) (2x2 Low) EVA70 | 10 000 | 20 000 |
| 2 | R.30-1 TDD + R.11-9 TDD (20+20+15 MHz, full,16QAM,1/2) (2x2 Low) EVA70 | 10 000 | 20 000 |

Table G.3A.5-7B: Minimum Test time TDD PDSCH Open Loop Spatial Multiplexing 2x2 4DL CA

| Clause 8.2.2.3.1_A.3 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | TDD |
| 1 | R.30-1 TDD (4x20 MHz, full,16QAM,1/2) (2x2 Low) EVA70 | 10 000 | 20 000 |
| 2 | R.30-1 TDD + R.11-9 TDD (20+20+20+15 MHz, full,16QAM,1/2) (2x2 Low) EVA70 | 10 000 | 20 000 |

Table G.3A.5-7C: Minimum Test time TDD PDSCH Open Loop Spatial Multiplexing 2x2 5DL CA

| Clause 8.2.2.3.1_A.4 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | TDD |
| 1 | R.30-1 TDD (5x20 MHz, full,16QAM,1/2) (2x2 Low) EVA70 | 10 000 | 20 000 |
| 2 | R.30-1 TDD + R.11-9 TDD (20+20+20+20+15 MHz, full,16QAM,1/2) (2x2 Low) EVA70 | 10 000 | 20 000 |

Table G.3A.5-8: Minimum Test time TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 2DL CA

| Clause 8.2.2.4.2_A.1 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | TDD |
| 1 | R.43TDD (2x20 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 100 000 |
| 2 | R.43TDD (20 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 100 000 |
|  | R.43-5 TDD (15 MHz, full, 16QAM,1/2) (4x2 Low) EVA 5 |  |  |

Table G.3A.5-8A: Minimum Test time TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 3DL CA

| Clause 8.2.2.4.2_A.2 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | TDD |
| 1 | R.43 TDD (3x20 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 100 000 |
| 2 | R.43 TDD + R.43-5 TDD (20+20+15 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 100 000 |

Table G.3A.5-8B: Minimum Test time TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 4DL CA

| Clause 8.2.2.4.2_A.3 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | TDD |
| 1 | R.43 TDD (4x20 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 100 000 |
| 2 | R.43 TDD + R.43-5 TDD (20+20+20+15 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 100 000 |

Table G.3A.5-8C: Minimum Test time TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 5DL CA

| Clause 8.2.2.4.2_A.4 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | TDD |
| 1 | R.43 TDD (5x20 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 100 000 |
| 2 | R.43 TDD + R.43-5 TDD (20+20+20+20+15 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 100 000 |

Table G.3A.5-9: Minimum Test time TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for Dual Connectivity

| Clause 8.2.2.4.4 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | TDD |
| 1 | R.43 TDD (2x20 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 100 000 |

Table G.3A.5-10Void

Table G.3A.5-11: Minimum Test time TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 (intra band non-contiguous DL CA)

| Clause 8.2.2.4.2_A.3 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | TDD |
| 1 | R.43TDD (2x20 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 100 000 |

Table G.3A.5-12: Minimum Test time FDD PDSCH Soft buffer management test (2 DL CA)

| Clause 8.2.1.3.1A_A.1 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD |
| 1 | R.30 FDD (2x20 MHz, full 16QAM, 1/2) (2x2 Low) EVA70 | 10 000 | 11 112 |
| 2 | R.35-2 FDD (15MHz, full 64QAM, 0.39) (2x2 Low) EVA5 | 10 000 | 12 500 |
|  | R.35-3 FDD (10MHz, full 64QAM, 0.39) (2x2 Low) EVA5 |  |  |
| 3 | R.30 FDD (20 MHz, full 16QAM, 1/2) (2x2 Low) EVA70 | 10 000 | 11 112 |
|  | R.11 FDD (10 MHz, full 16QAM, 1/2) (2x2 Low) EVA70 |  |  |
| 4 | R.30 FDD (20 MHz, full 16QAM, 1/2) (2x2 Low) EVA70 | 10 000 | 12 500 |
|  | R.30-1 FDD (15 MHz, full 16QAM, 1/2) (2x2 Low) EVA70 |  |  |
| 5 | R.35-1 FDD (2x20 MHz, full 64QAM, 0.39) (2x2 Low) EVA5 | 10 000 | 12 500 |
| 6 | R.35-1 FDD (20 MHz, full 64QAM, 0.39) (2x2 Low) EVA5 | 10 000 | 12 500 |
|  | R.35-3 FDD (10MHz, full 64QAM, 0.39) (2x2 Low) EVA5 |  |  |
| 7 | R.35-1 FDD (20 MHz, full 64QAM, 0.39) (2x2 Low) EVA5 | 10 000 | 12 500 |
|  | R.35-2 FDD (15MHz, full 64QAM, 0.39) (2x2 Low) EVA5 |  |  |

Table G.3A.5-14: Minimum Test time TDD PDSCH Soft buffer management test (2 DL CA)

| Clause 8.2.2.3.1A_A.1 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | TDD |
| 1 | R.30-2 TDD (2x20 MHz,full,16QAM,1/3) (2x2 Low) EVA70 | 10 000 | 50 000 |
| 2 | R.35-1 TDD (2x20 MHz,full,64QAM,1/3) (2x2 Low) EVA5 | 10 000 | 50 000 |

Table G.3A.5-15: Minimum Test time TDD power imbalance test (2 DL CA)

| Clause 8.2.2.7_A.1 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes | Minimum Number of Subframes (MNS) in the measured CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | TDD |
| 1 | R.49 TDD for PCC (20 MHz,full, 64QAM, 0.81-087) (1x2) OCNG for SCC | 10 000 in PCC | 20 000 |
| 2 | OCNG for PCC R.49-1 TDD for SCC (15 MHz,full, 64QAM, 0.80-0.86) (1x2) | 10 000 in SCC | 20 000 |

Table G.3A.5-15A: Minimum Test time TDD Intra-band contiguous carrier aggregation with minimum channel spacing (2DL CA)

| Clause 8.2.2.8.1 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes | Minimum Number of Subframes (MNS) in the measured CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | TDD |
| 1 | R.9 TDD for PCC and SCC (20 MHz,full, 64QAM, 3/4) (1x2 Low) EVA, 5 | 100 000 in PCC and SCC | 200 000 |

Table G.3A.5-15B: Minimum Test time TDD Intra-band contiguous carrier aggregation with minimum channel spacing (3DL CA)

| Clause 8.2.2.8.2 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes | Minimum Number of Subframes (MNS) in the measured CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | TDD |
| 1 | R.9 TDD for PCC and SCC (20 MHz,full, 64QAM, 3/4) (1x2 Low) EVA, 5 | 100 000 in PCC and SCC | 200 000 |

Table G.3A.5-16: Minimum Test time TDD FDD CA PDSCH Single Antenna Port Performance for FDD Pcell (2DL CA)

| Clause 8.2.3.1.1.1 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in the measured CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.42 FDD+R.42 TDD (20+20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 2 | R.2 FDD+R.42 TDD (20+10 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 3 | R.42-3 FDD+R.42 TDD (20+15 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |

Table G.3A.5-17: Minimum Test time TDD FDD CA PDSCH Single Antenna Port Performance for FDD PCell (3DL CA)

| Clause 8.2.3.1.1.2 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in the measured CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.42 FDD+R.42 TDD (20+2x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 2 | R.42-3 FDD+R.42 TDD (15+2x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 3 | R.2 FDD+R.42 TDD (10+2x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 4 | R.42 FDD+R.42 TDD (2x20+20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 5 | R.42-3 FDD+R.42 FDD+R.42 TDD (15+20+20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 6 | R.2 FDD+R.42 FDD +R.42 TDD (10+20+20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |

Table G.3A.5-17a: Minimum Test time TDD FDD CA PDSCH Single Antenna Port Performance for FDD PCell (4DL CA)

| Clause 8.2.3.1.1.3 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in the measured CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.42 FDD+R.42 TDD (20+3x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 2 | R.42 FDD+R.42 TDD (2x20+2x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 3 | R.42 FDD+R.42-3 FDD+ R.42 TDD (20+15+2x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 4 | R.42-3 FDD+R.42 TDD (2x15+2x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 5 | R.42 FDD +R.42-3 FDD+R.42 FDD+R.42 TDD (2x20+15+20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 6 | R.42 FDD+ R.42-3 FDD +R.42 TDD (2x15+20+20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 7 | R.42 FDD+ R.2 FDD +R.42 TDD (2x20+10+20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |

Table G.3A.5-17b: Minimum Test time TDD FDD CA PDSCH Single Antenna Port Performance for FDD PCell (5DL CA)

| Clause 8.2.3.1.1.4 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in the measured CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.42-3 FDD+R.42 FDD+R.42 TDD (15+2x20+2x20MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 2 | R.42-3 FDD+R.42 FDD+R.42 TDD (2x15+20+2x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |

Table G.3A.5-17c: Minimum Test time TDD FDD CA PDSCH Single Antenna Port Performance for FDD PCell (6DL CA)

| Clause 8.2.3.1.1.5 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in the measured CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.42 FDD+R.42 TDD (20+5x20MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 2 | R.42 FDD+R.42 TDD (2x20+4x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |

Table G.3A.5-17d: Minimum Test time TDD FDD CA PDSCH Single Antenna Port Performance for FDD PCell (7DL CA)

| Clause 8.2.3.1.1.6 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in the measured CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.42 FDD+R.42 TDD (20+6x20MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 2 | R.42 FDD+R.42 TDD (2x20+5x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |

Table G.3A.5-18: Minimum Test time TDD FDD CA PDSCH Single Antenna Port Performance for TDD Pcell (2DL CA)

| Clause 8.2.3.1.2.1 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in the measured CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.42 FDD+R.42 TDD (20+20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 2 | R.2 FDD+R.42 TDD (20+10 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 3 | R.42-3 FDD+R.42 TDD (20+15 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |

Table G.3A.5-19: Minimum Test time TDD FDD CA PDSCH Single Antenna Port Performance for TDD PCell(3DL CA)

| Clause 8.2.3.1.2.2 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in the measured CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.42 FDD+R.42 TDD (20+2x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 2 | R.42-3 FDD+R.42 TDD (15+2x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 3 | R.2 FDD+R.42 TDD (10+2x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 4 | R.42 FDD+R.42 TDD (2x20+20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 5 | R.42-3 FDD+R.42 FDD+R.42 TDD (15+20+20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 6 | R.2 FDD+R.42 FDD +R.42 TDD (10+20+20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |

Table G.3A.5-19a: Minimum Test time TDD FDD CA PDSCH Single Antenna Port Performance for TDD PCell(4DL CA)

| Clause 8.2.3.1.2.3 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in the measured CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.42 FDD+R.42 TDD (20+3x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 2 | R.42 FDD+R.42 TDD (2x20+2x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 3 | R.42 FDD+ R.42-3 FDD+ R.42 TDD (20+15+2x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 4 | R.42-3 FDD+R.42 TDD (2x15+2x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 5 | R.42 FDD + R.42-3 FDD+R.42 TDD (2x20+15+20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 6 | R.42-3 FDD+R.42 FDD +R.42 TDD (2x15+20+20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 7 | R.42 FDD+R.2 FDD +R.42 TDD (2x20+10+20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |

Table G.3A.5-19b: Minimum Test time TDD FDD CA PDSCH Single Antenna Port Performance for TDD PCell(5DL CA)

| Clause 8.2.3.1.2.4 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in the measured CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.42-3 FDD+ R.42 FDD+R.42 TDD (15+2x20+2x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 2 | R.42-3 FDD+ R.42 FDD + R.42 TDD (2x15+20+2x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |

Table G.3A.5-19c: Minimum Test time TDD FDD CA PDSCH Single Antenna Port Performance for TDD PCell (6DL CA)

| Clause 8.2.3.1.2.5 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in the measured CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.42 FDD+R.42 TDD (20+5x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 2 | R.42 FDD + R.42 TDD (2x20+4x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |

Table G.3A.5-19d: Minimum Test time TDD FDD CA PDSCH Single Antenna Port Performance for TDD PCell (7DL CA)

| Clause 8.2.3.1.2.6 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in the measured CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.42 FDD+R.42 TDD (20+6x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |
| 2 | R.42 FDD + R.42 TDD (2x20+5x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 100 000 |

Table G.3A.5-20: Minimum Test time TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for FDD PCell (2DL CA)

| Clause 8.2.3.2.1.1 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in the measured CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.30 FDD + R.30-1 TDD (20+20 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |
| 2 | R.11 FDD + R.30-1 TDD (20+10 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |
| 3 | R.11-7 FDD + R.30-1 TDD (20+15 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |

Table G.3A.5-21: Minimum Test time TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for FDD PCell (3DL CA)

| Clause 8.2.3.2.1.2 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in the measured CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.30 FDD + R.30-1 TDD (20+2x20 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |
| 2 | R.11-7 FDD + R.30-1 TDD (15+2x20 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |
| 3 | R.11 FDD + R.30-1 TDD (10+2x20 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |

Table G.3A.5-21a: Minimum Test time TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for FDD PCell (4DL CA)

| Clause 8.2.3.2.1.3 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in the measured CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.30 FDD + R.30-1 TDD (20+3x20 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |
| 2 | R.30FDD + R.30-1 TDD (2x20+2x20 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |
| 3 | R.30FDD +R.11-7 FDD + R.30-1 TDD (20+15+2x20 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |
| 4 | R.11-7 FDD + R.30FDD + R.30-1 TDD (2x15+2x20 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |
| 5 | R.30FDD +R.11-7 FDD + R.30-1 TDD (2x20+15+20 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |
| 6 | R.11-7 FDD + R.30FDD +R.30-1 TDD (2x15+20+20 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |
| 7 | R.30FDD +R.11 FDD + R.30-1 TDD (2x20+10+20 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |

Table G.3A.5-21b: Minimum Test time TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for FDD PCell (5DL CA)

| Clause 8.2.3.2.1.4 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in the measured CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.11-7 FDD+R.30 FDD + R.30-1 TDD (15+2x20+2x20 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |
| 2 | R.11-7 FDD+R.30FDD + R.30-1 TDD (2x15+20+2x20 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |

Table G.3A.5-21c: Minimum Test time TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for FDD PCell (6DL CA)

| Clause 8.2.3.2.1.5 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in the measured CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.30 FDD + R.30-1 TDD (1x20+5x20 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |
| 2 | R.30FDD + R.30-1 TDD (2x20+4x20 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |

Table G.3A.5-21d: Minimum Test time TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for FDD PCell (7DL CA)

| Clause 8.2.3.2.1.6 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in the measured CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.30 FDD + R.30-1 TDD (1x20+6x20 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |
| 2 | R.30FDD + R.30-1 TDD (2x20+5x20 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |

Table G.3A.5-22: Minimum Test time TDD FDD CA Soft buffer management test for FDD PCell (2DL CA)

| Clause 8.2.3.2.1A Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in the measured CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.35-1 FDD+ R.35-1 TDD (20+20 MHz,full,64QAM,0.39) (2x2 Low) EVA70 | 10 000 | 50 000 |
| 2 | R.35-1 FDD+ R.35-1 TDD (20+20 MHz,full,64QAM,0.39) (2x2 Low) EVA70 | 10 000 | 50 000 |
| 3 | R.35-3 FDD+ R.35-1 TDD (10+20 MHz,full,64QAM,0.39) (2x2 Low) EVA70 | 10 000 | 50 000 |
| 4 | R.35-3 FDD+ R.35-1 TDD (10+20 MHz,full,64QAM,0.39) (2x2 Low) EVA70 | 10 000 | 50 000 |
| 5 | R.35-2 FDD+ R.35-1 TDD (15+20 MHz,full,64QAM,0.39) (2x2 Low) EVA70 | 10 000 | 50 000 |
| 6 | R.35-2 FDD+ R.35-1 TDD (15+20 MHz,full,64QAM,0.39) (2x2 Low) EVA70 | 10 000 | 50 000 |

Table G.3A.5-23: Minimum Test time TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for TDD PCell (2DL CA)

| Clause 8.2.3.2.2.1 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in the measured CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.30 FDD + R.30-1 TDD (20+20 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |
| 2 | R.11 FDD + R.30-1 TDD (20+10 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |
| 3 | R.11-7 FDD + R.30-1 TDD (20+15 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |

Table G.3A.5-24: Minimum Test time TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for TDD PCell (3DL CA)

| Clause 8.2.3.2.2.2 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in the measured CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.30 FDD + R.30-1 TDD (20+2x20 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |
| 2 | R.11-7 FDD + R.30-1 TDD (15+2x20 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |
| 3 | R.11 FDD + R.30-1 TDD (10+2x20 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |

Table G.3A.5-24a: Minimum Test time TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for TDD PCell (4DL CA)

| Clause 8.2.3.2.2.3 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in the measured CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.30 FDD + R.30-1 TDD (20+3x20 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |
| 2 | R.30 FDD + R.30-1 TDD (2x20+2x20MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |
| 3 | R.30FDD +R.11-7 FDD + R.30-1 TDD (20+15+2x20MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |
| 4 | R.11-7 FDD+ R.30FDD + R.30-1 TDD (2x15+2x20MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |
| 5 | R.30FDD +R.11-7 FDD + R.30-1 TDD (2x20+15+20MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |
| 6 | R.11-7 FDD+ R.30FDD+ R.30-1 TDD (2x15+20+20MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |
| 7 | R.30FDD +R.11 FDD + R.30-1 TDD (2x20+10+20 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |

Table G.3A.5-24b: Minimum Test time TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for TDD PCell (5DL CA)

| Clause 8.2.3.2.2.4 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in the measured CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.11-7 FDD+R.30 FDD + R.30-1 TDD (15+2x20+2x20 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |
| 2 | R.11-7 FDD+R.30FDD + R.30-1 TDD (2x15+20+2x20 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |

Table G.3A.5-24c: Minimum Test time TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for TDD PCell (6DL CA)

| Clause 8.2.3.2.2.5 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in the measured CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.30 FDD + R.30-1 TDD (20+5x20 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |
| 2 | R.30FDD + R.30-1 TDD (2x20+4x20 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |

Table G.3A.5-24d: Minimum Test time TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for TDD PCell (7DL CA)

| Clause 8.2.3.2.2.6 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in the measured CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.30 FDD + R.30-1 TDD (20+6x20 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |
| 2 | R.30FDD + R.30-1 TDD (2x20+5x20 MHz,full,QPSK,1/3) (2x2 Low) EVA70 | 10 000 | 25 000 |

Table G.3A.5-25: Minimum Test time TDD FDD CA PDSCH Soft buffer management test for TDD PCell (2DL CA)

| Clause 8.2.3.2.2A Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in the measured CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.35-1 FDD+ R.35-1 TDD (20+20 MHz,full,64QAM,0.39) (2x2 Low) EVA70 | 10 000 | 50 000 |
| 2 | R.35-1 FDD+ R.35-1 TDD (20+20 MHz,full,64QAM,0.39) (2x2 Low) EVA70 | 10 000 | 50 000 |
| 3 | R.35-3 FDD+ R.35-1 TDD (10+20 MHz,full,64QAM,0.39) (2x2 Low) EVA70 | 10 000 | 50 000 |
| 4 | R.35-3 FDD+ R.35-1 TDD (10+20 MHz,full,64QAM,0.39) (2x2 Low) EVA70 | 10 000 | 50 000 |
| 5 | R.35-2 FDD+ R.35-1 TDD (15+20 MHz,full,64QAM,0.39) (2x2 Low) EVA70 | 10 000 | 50 000 |
| 6 | R.35-2 FDD + R.35-1 TDD (15+20 MHz,full,64QAM,0.39) (2x2 Low) EVA70 | 10 000 | 50 000 |

Table G.3A.5-26: Minimum Test time TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for FDD PCell (2DL CA)

| Clause 8.2.3.3.1.1 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.14-3 FDD + R.43TDD (2x20 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 125 000 |
| 2 | R.14 FDD + R.43TDD (20+10 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 125 000 |
| 3 | R.14-7 FDD + R.43 TDD (20+15 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 125 000 |

Table G.3A.5-27: Minimum Test time TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for FDD PCell (3DL CA)

| Clause 8.2.3.3.1.2 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.14-3 FDD + R.43TDD (3x20 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 125 000 |
| 2 | R.14-7 FDD +R.43 TDD (20+20+15 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 125 000 |
| 3 | R.14 FDD +R.43 TDD (20+20+10 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 125 000 |

Table G.3A.5-27a: Minimum Test time TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for FDD PCell (4DL CA)

| Clause 8.2.3.3.1.3 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
| 1 | R.14-3 FDD+R.43 TDD (20+3x20 MHz, full,16QAM, 1/2) (4x2 Low) EVA5 | 50 000 | 125 000 |
| 2 | R.14-3 FDD+R.43 TDD (2x20+2x20 MHz, full,16QAM, 1/2) (4x2 Low) EVA5 | 50 000 | 125 000 |
| 3 | R.14-3FDD+ R.14-7FDD+R.43TDD (20+15+2x20 MHz, full,16QAM,1/2) (4x2 Low) EVA5 | 50 000 | 125 000 |
| 4 | R.14-7 FDD+R.43 TDD (2x15+2x20 MHz, full,16QAM, 1/2) (4x2 Low) EVA5 | 50 000 | 125 000 |
| 5 | R.14-3FDD +R.14-7FDD+R.43TDD (2x20+15+20 MHz, full,16QAM,1/2) (4x2 Low) EVA5 | 50 000 | 125 000 |
| 6 | R.14-7FDD+ R.14-3FDD+R.43TDD (2x15+20+20 MHz, full,16QAM,1/2) (4x2 Low) EVA5 | 50 000 | 125 000 |
| 7 | R.14-3 FDD+ R.14 FDD +R.43TDD (2x20+10+20 MHz, full,16QAM,1/2) (4x2 Low) EVA5 | 50 000 | 125 000 |

Table G.3A.5-27b: Minimum Test time TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for FDD PCell (5DL CA)

| Clause 8.2.3.3.1.4 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
| 1 | R.14-7 FDD+R.14-3FDD+R.43TDD (15+2x20+2x20MHz, full,16QAM,1/2) (4x2 Low) EVA5 | 50 000 | 125 000 |
| 2 | R.14-7 FDD+R.14-3FDD+R.43TDD (2x15+20+2x20 MHz, full,16QAM,1/2) (4x2 Low) EVA5 | 50 000 | 125 000 |

Table G.3A.5-27c: Minimum Test time TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for FDD PCell (6DL CA)

| Clause 8.2.3.3.1.5 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
| 1 | R.14-3FDD+R.43TDD (20+5x20MHz, full,16QAM,1/2) (4x2 Low) EVA5 | 50 000 | 125 000 |
| 2 | R.14-3FDD+R.43TDD (2x20+4x20 MHz, full,16QAM,1/2) (4x2 Low) EVA5 | 50 000 | 125 000 |

Table G.3A.5-27d: Minimum Test time TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for FDD PCell (7DL CA)

| Clause 8.2.3.3.1.6 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
| 1 | R.14-3FDD+R.43TDD (20+6x20MHz, full,16QAM,1/2) (4x2 Low) EVA5 | 50 000 | 125 000 |
| 2 | R.14-3FDD+R.43TDD (2x20+5x20 MHz, full,16QAM,1/2) (4x2 Low) EVA5 | 50 000 | 125 000 |

Table G.3A.5-28: Minimum Test time TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for TDD PCell (2DL CA)

| Clause 8.2.3.3.2.1 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.14-3 FDD + R.43TDD (2x20 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 125 000 |
| 2 | R.14 FDD + R.43TDD (20+10 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 125 000 |
| 3 | R.14-7 FDD + R.43TDD (15+20 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 125 000 |

Table G.3A.5-29: Minimum Test time TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for TDD PCell (3DL CA)

| Clause 8.2.3.3.2.2 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.14-3 FDD + R.43TDD (3x20 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 125 000 |
| 2 | R.14 FDD + R.43TDD (20+20+15 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 125 000 |
| 3 | R.14-7 FDD + R.43TDD (20+20+10 MHz,full,16QAM, 1/2) (4x2 Low) EVA 5 | 50 000 | 125 000 |

Table G.3A.5-29a: Minimum Test time TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for TDD PCell (4DL CA)

| Clause 8.2.3.3.2.3 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.14-3 FDD+R.43 TDD (20+3x20 MHz, full,16QAM, 1/2) (4x2 Low) EVA5 | 50 000 | 125 000 |
| 2 | R.14-3 FDD+R.43 TDD (2x20+2x20 MHz, full,16QAM, 1/2) (4x2 Low) EVA5 | 50 000 | 125 000 |
| 3 | R.14-3FDD+ R.14-7FDD+R.43TDD (20+15+2x20 MHz, full,16QAM,1/2) (4x2 Low) EVA5 | 50 000 | 125 000 |
| 4 | R.14-7 FDD+R.43 TDD (2x15+2x20 MHz, full,16QAM, 1/2) (4x2 Low) EVA5 | 50 000 | 125 000 |
| 5 | R.14-3FDD +R.14-7FDD+R.43TDD (2x20+15+20 MHz, full,16QAM,1/2) (4x2 Low) EVA5 | 50 000 | 125 000 |
| 6 | R.14-7FDD+ R.14-3FDD+R.43TDD (2x15+20+20 MHz, full,16QAM,1/2) (4x2 Low) EVA5 | 50 000 | 125 000 |
| 7 | R.14-3 FDD+ R.14 FDD +R.43TDD (2x20+10+20 MHz, full,16QAM,1/2) (4x2 Low) EVA5 | 50 000 | 125 000 |

Table G.3A.5-29b: Minimum Test time TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for TDD PCell (5DL CA)

| Clause 8.2.3.3.2.4 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
| 1 | R.14-7 FDD+R.14-3FDD+R.43TDD (15+2x20+2x20MHz, full,16QAM,1/2) (4x2 Low) EVA5 | 50 000 | 125 000 |
| 2 | R.14-7 FDD+R.14-3FDD+R.43TDD (2x15+20+2x20 MHz, full,16QAM,1/2) (4x2 Low) EVA5 | 50 000 | 125 000 |

Table G.3A.5-29c: Minimum Test time TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for TDD PCell (6DL CA)

| Clause 8.2.3.3.2.5 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
| 1 | R.14-3FDD+R.43TDD (20+5x20MHz, full,16QAM,1/2) (4x2 Low) EVA5 | 50 000 | 125 000 |
| 2 | R.14-3FDD+R.43TDD (2x20+4x20 MHz, full,16QAM,1/2) (4x2 Low) EVA5 | 50 000 | 125 000 |

Table G.3A.5-29d: Minimum Test time TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for TDD PCell (7DL CA)

| Clause 8.2.3.3.2.6 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
| 1 | R.14-3FDD+R.43TDD (20+6x20MHz, full,16QAM,1/2) (4x2 Low) EVA5 | 50 000 | 125 000 |
| 2 | R.14-3FDD+R.43TDD (2x20+5x20 MHz, full,16QAM,1/2) (4x2 Low) EVA5 | 50 000 | 125 000 |

Table G.3A.5-30: LAA Dual-Layer Spatial Multiplexing with DM-RS with FDD as Pcell

| Clause 8.3.3.1.1 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD |
| 1 | R.30 FDD (1x20 MHz, full 16QAM, 1/2), (2x2 Low) EVA70 + R.2-FS3 (1x20 MHz, full 16QAM, 1/2), (2x2 Low) EVA5 | 50,000 | 121,000 |
| 2 | R.30 FDD (1x20 MHz, full 16QAM, 1/2), (2x2 Low) EVA70 + R.2-FS3 (1x20 MHz, full 16QAM, 1/2), (2x2 Low) EVA5 | 50,000 | 121,000 |
| 3 | R.30 FDD (1x20 MHz, full 16QAM, 1/2), (2x2 Low) EVA70 + R.2-FS3 (1x20 MHz, full 16QAM, 1/2), (2x2 Low) EVA5 | 50,000 | 121,000 |
| 4 | R.30 FDD (1x20 MHz, full 16QAM, 1/2), (2x2 Low) EVA70 + R.2-FS3 (1x20 MHz, full 16QAM, 1/2), (2x2 Low) EVA5 | 50,000 | 121,000 |
| NOTE 1: MNAS is not simulated, but estimated based on similar scenario in Table G.3A.5-16 Test1. NOTE 2: MNS for SCC is not deterministic due to the statistical burst transmission model in clause B.3.8. The average MNS is presented in this table. This is calculated based on an average burst length of 5.25 ms which in average takes 6.25 ms assuming UE not capable of partial subframes, and a simulated probability of LBT success (p) equal to 0.5 (ratio 6.25/5.25/0.5). |  |  |  |

Table G.3A.5-30A: LAA Dual-Layer Spatial Multiplexing with DM-RS with TDD as Pcell

| Clause 8.3.3.1.2 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | TDD |
| 1 | R.30-1 TDD (1x20 MHz, full 16QAM, 1/2), (2x2 Low) EVA70 + R.2 FS3 (1x20 MHz, full 16QAM, 1/2), (2x2 Low) EVA5 | 50,000 | 121,000 |
| 2 | R.30-1 TDD (1x20 MHz, full 16QAM, 1/2), (2x2 Low) EVA70 + R.2 FS3 (1x20 MHz, full 16QAM, 1/2), (2x2 Low) EVA5 | 50,000 | 121,000 |
| 3 | R.30-1 TDD (1x20 MHz, full 16QAM, 1/2), (2x2 Low) EVA70 + R.2 FS3 (1x20 MHz, full 16QAM, 1/2), (2x2 Low) EVA5 | 50,000 | 121,000 |
| 4 | R.30-1 TDD (1x20 MHz, full 16QAM, 1/2), (2x2 Low) EVA70 + R.2 FS3 (1x20 MHz, full 16QAM, 1/2), (2x2 Low) EVA5 | 50,000 | 121,000 |
| NOTE 1: MNAS is not simulated, but estimated based on similar scenario in Table G.3A.5-16 Test1. NOTE 2: MNS for SCC is not deterministic due to the statistical burst transmission model in clause B.3.8. The average MNS is presented in this table. This is calculated based on an average burst length of 5.25 ms which in average takes 6.25 ms assuming UE not capable of partial subframes, and a simulated probability of LBT success (p) equal to 0.5 (ratio 6.25/5.25/0.5). |  |  |  |

Table G.3A.5-31: Minimum Test time LAA PDSCH CA Closed Loop Spatial Multiplexing Performance with 4Tx Antenna ports and with FDD as PCell (FDD Single Carrier)

| Clause 8.2.4.1.1 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/FS3 |
| 1 | R.14-3 FDD + R.1 FS3 (20 MHz, full,16QAM, 1/2) + (20 MHz, full, 64QAM, 0.6) (4x2 Low) + (4x2 Low) EVA 5 + EVA5 | 50 000 | 300,000 |
| NOTE 1: MNAS is not simulated, but estimated based on similar scenario in Table G.3A.5-xx Test1. NOTE 2: MNS for SCC is not deterministic due to the statistical burst transmission model in clause B.3.8. The average MNS is presented in this table. This is calculated based on an average burst length of 5.25 ms which in average takes 6.25 ms assuming UE not capable of partial subframes, and a simulated probability of LBT success (p) equal to 0.5 (ratio 6.25/5.25/0.5). |  |  |  |

Table G.3A.5-32: Minimum Test time LAA PDSCH CA Closed Loop Spatial Multiplexing Performance with 4Tx Antenna ports and with TDD as PCell (TDD Single Carrier)

| Clause 8.2.4.1.2 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | TDD/FS3 |
| 1 | R.43 TDD + R.1 FS3 (20 MHz, full,16QAM, 1/2) + (20 MHz, full, 64QAM, 0.6) (4x2 Low) + (4x2 Low) EVA 5 + EVA5 | 50 000 | 300 000 |
| NOTE 1: MNAS is not simulated, but estimated based on similar scenario in Table G.3A.5-yy Test1. NOTE 2: MNS for SCC is not deterministic due to the statistical burst transmission model in clause B.3.8. The average MNS is presented in this table. This is calculated based on an average burst length of 5.25 ms which in average takes 6.25 ms assuming UE not capable of partial subframes, and a simulated probability of LBT success (p) equal to 0.5 (ratio 6.25/5.25/0.5). |  |  |  |

Table G.3A.5-33: Minimum Test time for PDSCH Dual-layer Spatial Multiplexing for 4Rx (2DL CA - 5DL CA)

| Clause 8.13.1.2.2-8.13.1.2.5/8.13.2.2.2-8.13.2.2.5/8.13.3.2.3-8.13.3.2.10 Test No | Demodulation scenario (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| All | R.51 (5-20MHz, full, 16QAM,) (2x4 Low) ETU5 | 50000 Note: MNAS is not simulated. It is estimated based on other similar test cases using EPA5 propagation conditions | 56 000 | 100 000 |

Table G.3.5A-34: Minimum Test time for PDSCH Closed Loop Multi Layer Spatial Multiplexing for 4Rx (2DL CA - 5DL CA)

| Clause 8.13.1.1.1.2-8.13.1.1.1.5/8.13.2.1.1.2-8.13.2.1.1.5 Test No | Demodulation scenario (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| All | R.43 (1.4-20MHz, full, 16QAM,) (4x4 Low) EVA5 | 50000 Note: MNAS is not simulated. It is estimated based on other similar test cases using EVA5 propagation conditions | 56 000 | 100 000 |

Table G.3A.5-35: Minimum Test time TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 (2DL CA - 5DL CA)

| Clause 8.13.3.1.1.2-8.13.3.1.1.5/ 8.13.3.1.2.2-8.13.3.1.2.5 Test No | Demodulation scenario (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| All | R.14 FDD + R.43 TDD (1.4-20MHz, full, 16QAM,) (4x4 Low) EVA5 | 50 000 | 125 000 |

Table G.3A.5-36: Minimum Test time TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 with 256QAM for FDD PCell (2DL CA)

| Clause 8.13.3.6.1 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.72-3 FDD + R.72-3 TDD (2x20 MHz,full,256QAM, 0.62) (4x4 Low) EPA 5 | 50 000 Note: MNAS is not simulated. It is estimated based on other similar test cases using EPA5 propagation conditions | 126 000 |
| 2 | R.72 FDD + R.72-3 TDD (10 + 20MHz,full,256QAM, 0.62) (4x4 Low) EPA 5 | 50 000 Note: MNAS is not simulated. It is estimated based on other similar test cases using EPA5 propagation conditions | 126 000 |
| 3 | R.72-2 FDD + R.72-3 TDD (15 MHz,full,256QAM, 0.61) + (20 MHz,full,256QAM, 0.62) (4x4 Low) EPA 5 | 50 000 Note: MNAS is not simulated. It is estimated based on other similar test cases using EPA5 propagation conditions | 126 000 |

Table G.3A.5-37: Minimum Test time TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 with 256QAM for TDD PCell (2DL CA)

| Clause 8.13.3.6.2 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| 1 | R.72-3 FDD + R.72-3 TDD (2x20 MHz,full,256QAM, 0.62) (4x4 Low) EPA 5 | 50 000 Note: MNAS is not simulated. It is estimated based on other similar test cases using EPA5 propagation conditions | 126 000 |
| 2 | R.72 FDD + R.72-3 TDD (10 + 20MHz,full,256QAM, 0.62) (4x4 Low) EPA 5 | 50 000 Note: MNAS is not simulated. It is estimated based on other similar test cases using EPA5 propagation conditions | 126 000 |
| 3 | R.72-2 FDD + R.72-3 TDD (15 MHz,full,256QAM, 0.61) + (20 MHz,full,256QAM, 0.62) (4x4 Low) EPA 5 | 50 000 Note: MNAS is not simulated. It is estimated based on other similar test cases using EPA5 propagation conditions | 126 000 |

Table G.3A.5-38: Minimum Test time for PDSCH Closed Loop Multi Layer Spatial Multiplexing with 256QAM (2DL CA)

| Clause 8.13.1.1.3/ 8.13.2.1.3 Test No | Demodulation scenario (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| All | R.72 (1.4-20MHz, full, 256QAM,) (4x4 Low) EPA5 | 50000 | 56 000 | 100 000 |

Table G.3A.5-39: Minimum Test time for PDSCH Closed Loop Four-Layer Spatial Multiplexing (2DL CA)

| Clause 8.13.1.1.4/ 8.13.2.1.4 Test No | Demodulation scenario (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| All | R.74 (1.4-20MHz, full, 16QAM,) (4x4 Low) EPA5 | 50000 | 56 000 | 100 000 |

Table G.3.5A-40: Minimum Test time for TDD-FDD CA PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model-Enhanced Performance Requirement Type A (2DL CA)

| Clause 8.13.3.3.1/ 8.13.3.3.2 Test No | Demodulation scenario (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| All | R.47 FDD + R.47 TDD (5-20MHz, full, 16QAM) (2x4 Low) EVA5 | 50 000 | 125 000 |

Table G.3.5A-41: Minimum Test time for TDD-FDD CA PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 Interference Model-Enhanced Performance Requirement Type A (2DL CA)

| Clause 8.13.3.4.1/ 8.13.3.4.2 Test No | Demodulation scenario (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| All | R.76 FDD + R.76 TDD (5-20MHz, full, QPSK) (2x4 Low) EVA5 | 50 000 | 125 000 |

Table G.3.5A-42: Minimum Test time for 4-Rx PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model – Enhanced Performance Type A(2DL CA)

| Clause 8.13.1.3.1 8.13.2.3.1 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.47 (2x10 MHz, full, 16QAM, 1/3) (2x2+ 2x4 Low) EVA5 | Note: MNAS is not simulated. It is estimated based on other similar test cases using EVA5 propagation conditions | 56 000 | 100 000 |
| 2 | R.47-3 (2x20 MHz, full, 16QAM, 1/3) (2x2+ 2x4 Low) EVA5 | Note: MNAS is not simulated. It is estimated based on other similar test cases using EVA5 propagation conditions | 56 000 | 100 000 |
| 3 | R.47-1 (2x5 MHz, full, 16QAM, 1/3) (2x2+ 2x4 Low) EVA5 | Note: MNAS is not simulated. It is estimated based on other similar test cases using EVA5 propagation conditions | 56 000 | 100 000 |
| 4 | R.47-2 + R.47-1 (15+5 MHz, full, 16QAM, 1/3) (2x2+ 2x4 Low) EVA5 | Note: MNAS is not simulated. It is estimated based on other similar test cases using EVA5 propagation conditions | 56 000 | 100 000 |
| 5 | R.47 + R.47-1 (10+5 MHz, full, 16QAM, 1/3) (2x2+ 2x4 Low) EVA5 | Note: MNAS is not simulated. It is estimated based on other similar test cases using EVA5 propagation conditions | 56 000 | 100 000 |

Table G.3.5A-43: Minimum Test time for 4-Rx PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 with TM9 Interference Model – Enhanced Performance Type A(2DL CA)

| Clause 8.13.1.4.1 8.13.2.4.1 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | R.76 + R76-5 (2x10 MHz, full, QPSK, 1/2) (2x2+ 2x4 Low) EVA5 | Note: MNAS is not simulated. It is estimated based on other similar test cases using EVA5 propagation conditions | 56 000 | 100 000 |
| 2 | R.76-3+ R.76-7 (2x20 MHz, full, QPSK, 1/2) (2x2+ 2x4 Low) EVA5 | Note: MNAS is not simulated. It is estimated based on other similar test cases using EVA5 propagation conditions | 56 000 | 100 000 |
| 3 | R.76-1+ R.76-4 (2x5 MHz, full, QPSK, 1/2) (2x2+ 2x4 Low) EVA5 | Note: MNAS is not simulated. It is estimated based on other similar test cases using EVA5 propagation conditions | 56 000 | 100 000 |
| 4 | R.76-2+R.76-6+ R.76-1+R.76-4 (15+5 MHz, full, QPSK, 1/2) (2x2+ 2x4 Low) EVA5 | Note: MNAS is not simulated. It is estimated based on other similar test cases using EVA5 propagation conditions | 56 000 | 100 000 |
| 5 | R.76+R.76-5+ R.76-1+R.76-4 (10+5 MHz, full, QPSK, 1/2) (2x2+ 2x4 Low) EVA5 | Note: MNAS is not simulated. It is estimated based on other similar test cases using EVA5 propagation conditions | 56 000 | 100 000 |

Table G.3A.5-44: Minimum Test time TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 (2DL CA)

| Clause 8.13.7.1/ 8.13.7.2 Test No | Demodulation scenario (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD/TDD |
| All | R.74 (1.4-20MHz, full, 16QAM,) (4x4 Low) EPA5 | 50 000 | 125 000 |

## G.3A.6	Test conditions

Table G.3A.6-1: Test conditions for CA/DC performance tests

| Test | Statistical independence | Number of components in the test vector, as specified in the test requirements and initial conditions of the applicable test | Over all Pass/Fail condition |
| --- | --- | --- | --- |
| 8.2.1.1.1_A.1 FDD PDSCH Single Antenna Port Performance for CA (2DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.1.1_A.2 FDD PDSCH Single Antenna Port Performance for CA (3DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.1.1_A.4 FDD PDSCH Single Antenna Port Performance for CA (4DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.1.1_A.5 FDD PDSCH Single Antenna Port Performance for CA (5DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.1.1_A.6 FDD PDSCH Single Antenna Port Performance for CA (6DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.1.1_A.7 FDD PDSCH Single Antenna Port Performance for CA (7DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.3.1_A.1 FDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (2DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.3.1_A.2 FDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (3DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.3.1_A.3 FDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (4DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.3.1_A.4 FDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (5DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.3.1_A.5 FDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (6DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.3.1_A.6 FDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (7DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.3.1A_A.1 FDD PDSCH Soft buffer management test for CA (2 DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.4.2_A.1 FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4 x 2 for CA (2DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.4.2_A.2 FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4 x 2 for CA (3DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.4.2_A.3 FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4 x 2 for CA (4DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.4.2_A.4 FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4 x 2 for CA (5DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.4.2_A.5 FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (6DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.4.2_A.6 FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (7DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.4.3A FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for Dual Connectivity | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.2.1.1_A.1 TDD PDSCH Single Antenna Port Performance for CA (2DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.2.1.1_A.2 TDD PDSCH Single Antenna Port Performance for CA (3DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.2.1.1_A.3 TDD PDSCH Single Antenna Port Performance for CA (4DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.2.1.1_A.4 TDD PDSCH Single Antenna Port Performance for CA (5DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.2.3.1_A.1 TDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (2DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.2.3.1_A.2 TDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (3DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.2.3.1_A.3 TDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (4DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.2.3.1_A.4 TDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (5DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.2.3.1A_A.1 TDD PDSCH Soft buffer management test (2 DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.2.4.2_A.1 TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (2DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.2.4.2_A.2 TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (3DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.2.4.2_A.3 TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (4DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.2.4.2_A.4 TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (5DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.2.4.4 TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for Dual Connectivity | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.3.1.1.5 TDD FDD CA PDSCH Single Antenna Port Performance for FDD PCell (6DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.3.1.2.5 TDD FDD CA PDSCH Single Antenna Port Performance for TDD PCell (6DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.3.1.2.6 TDD FDD CA PDSCH Single Antenna Port Performance for TDD PCell (7DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.3.2.1.5 TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for FDD PCell (6DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.3.2.1.6 TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for FDD PCell (7DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.3.2.2.1 TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for TDD PCell (2DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.3.2.2.2 TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for TDD PCell (3DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.3.2.2.5 TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for TDD PCell (6DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.3.2.2.6 TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for TDD PCell (7DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.3.3.1.5 TDD FDD CA PDSCH Closed Loop Multi Lyaer Spatial Multiplexing 4x2 for FDD PCell (6DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.3.3.1.6 TDD FDD CA PDSCH Closed Loop Multi Lyaer Spatial Multiplexing 4x2 for FDD PCell (7DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.3.3.2.5 TDD FDD CA PDSCH Closed Loop Multi Lyaer Spatial Multiplexing 4x2 for TDD PCell (6DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.3.3.2.6 TDD FDD CA PDSCH Closed Loop Multi Lyaer Spatial Multiplexing 4x2 for TDD PCell (7DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.3.3.1.1 LAA Dual-Layer Spatial Multiplexing with DM-RS with FDD as Pcell | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.3.3.1.2 LAA Dual-Layer Spatial Multiplexing with DM-RS with TDD as Pcell | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.7.1.1_A.1 FDD Sustained data rate performance for CA (2DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.7.1.1_A.2 FDD Sustained data rate performance for CA (3DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.7.1.1_A.4 FDD Sustained data rate performance for CA (4DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.7.2.1_A.1 TDD Sustained data rate performance for CA (2DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.7.2.1_A.2 TDD Sustained data rate performance for CA (3DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.7.2.1_A.4 TDD Sustained data rate performance for CA (4DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.7.2.1_A.5 TDD Sustained data rate performance for CA (5DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 9.6.1.1_A.3 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 for CA (4DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |

# G.4	Statistical testing of Performance Requirements with probability of misdetection

## G.4.1	General

The test of receiver performance characteristics is two fold.

1.	A signal or a combination of signals is offered to the RX port(s) of the receiver.

2.	The ability of the receiver to demodulate /decode this signal is verified by analyzing the reaction of the UE to this signal.

In (2) is the statistical aspect of the test and is treated here.

The minimum requirement for those receiver performance tests are 1% or 0.1% misdetection probability

All receiver performance tests are performed in fading conditions. In addition to the statistical considerations, this requires the definition of a minimum test time.

NOTE:	All demodulation performance tests (state from version 9.5.0) require a minimum test time, which exceeds the maximum test time in tables G.4.4. Under this circumstances only the test limit at the end of tables G.4.4.-1 resp. G.4.4.-2 is applicable.

## G.4.2	Mapping the UE reaction to error ratio

The UE can not indicate the detection or misdetection of the physical channel under test directly. Indirect methods are described in the procedure of the applicable test.

## G.4.3	Design of the test

G.2.3 applies, exception:

Limit ER = 0.01 and ER = 0.001

## G.4.4	Numerical definition of the pass fail limits

Table G.4.4-1: pass fail limits for ER = 0.01

| ne | nsp | nsf | ne | nsp | nsf | ne | nsp | nsf | ne | nsp | nsf |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 344 | NA | 40 | 3929 | 2553 | 80 | 7033 | 5874 | 120 | 10036 | 9354 |
| 1 | 485 | NA | 41 | 4009 | 2632 | 81 | 7109 | 5960 | 121 | 10110 | 9442 |
| 2 | 607 | 10 | 42 | 4089 | 2712 | 82 | 7185 | 6046 | 122 | 10184 | 9530 |
| 3 | 719 | 33 | 43 | 4168 | 2792 | 83 | 7261 | 6131 | 123 | 10259 | 9619 |
| 4 | 826 | 66 | 44 | 4247 | 2873 | 84 | 7336 | 6217 | 124 | 10333 | 9707 |
| 5 | 929 | 107 | 45 | 4327 | 2953 | 85 | 7412 | 6303 | 125 | 10407 | 9796 |
| 6 | 1029 | 152 | 46 | 4406 | 3034 | 86 | 7488 | 6389 | 126 | 10481 | 9884 |
| 7 | 1127 | 202 | 47 | 4484 | 3115 | 87 | 7564 | 6475 | 127 | 10555 | 9972 |
| 8 | 1223 | 255 | 48 | 4563 | 3196 | 88 | 7639 | 6561 | 128 | 10629 | 10061 |
| 9 | 1317 | 311 | 49 | 4642 | 3278 | 89 | 7715 | 6648 | 129 | 10703 | 10150 |
| 10 | 1409 | 370 | 50 | 4720 | 3359 | 90 | 7790 | 6734 | 130 | 10777 | 10238 |
| 11 | 1501 | 430 | 51 | 4799 | 3441 | 91 | 7866 | 6820 | 131 | 10851 | 10327 |
| 12 | 1592 | 492 | 52 | 4877 | 3523 | 92 | 7941 | 6907 | 132 | 10925 | 10416 |
| 13 | 1681 | 555 | 53 | 4955 | 3605 | 93 | 8017 | 6993 | 133 | 10999 | 10504 |
| 14 | 1770 | 620 | 54 | 5033 | 3688 | 94 | 8092 | 7080 | 134 | 11073 | 10593 |
| 15 | 1858 | 686 | 55 | 5111 | 3770 | 95 | 8167 | 7167 | 135 | 11147 | 10682 |
| 16 | 1946 | 754 | 56 | 5189 | 3853 | 96 | 8242 | 7253 | 136 | 11221 | 10771 |
| 17 | 2032 | 822 | 57 | 5267 | 3935 | 97 | 8317 | 7340 | 137 | 11295 | 10860 |
| 18 | 2119 | 891 | 58 | 5344 | 4018 | 98 | 8393 | 7427 | 138 | 11369 | 10949 |
| 19 | 2204 | 961 | 59 | 5422 | 4101 | 99 | 8468 | 7514 | 139 | 11442 | 11038 |
| 20 | 2290 | 1032 | 60 | 5499 | 4185 | 100 | 8543 | 7601 | 140 | 11516 | 11127 |
| 21 | 2374 | 1103 | 61 | 5577 | 4268 | 101 | 8618 | 7688 | 141 | 11590 | 11216 |
| 22 | 2459 | 1175 | 62 | 5654 | 4352 | 102 | 8693 | 7775 | 142 | 11664 | 11305 |
| 23 | 2543 | 1248 | 63 | 5731 | 4435 | 103 | 8768 | 7863 | 143 | 11737 | 11394 |
| 24 | 2627 | 1321 | 64 | 5809 | 4519 | 104 | 8843 | 7950 | 144 | 11811 | 11483 |
| 25 | 2710 | 1395 | 65 | 5886 | 4603 | 105 | 8917 | 8037 | 145 | 11885 | 11573 |
| 26 | 2793 | 1470 | 66 | 5963 | 4687 | 106 | 8992 | 8125 | 146 | 11958 | 11662 |
| 27 | 2876 | 1544 | 67 | 6039 | 4771 | 107 | 9067 | 8212 | 147 | 12032 | 11751 |
| 28 | 2958 | 1620 | 68 | 6116 | 4855 | 108 | 9142 | 8300 | 148 | 12105 | 11840 |
| 29 | 3040 | 1696 | 69 | 6193 | 4940 | 109 | 9216 | 8387 | 149 | 12179 | 11930 |
| 30 | 3122 | 1772 | 70 | 6270 | 5024 | 110 | 9291 | 8475 | 150 | 12252 | 12019 |
| 31 | 3204 | 1848 | 71 | 6346 | 5109 | 111 | 9366 | 8562 | 151 | 12326 | 12109 |
| 32 | 3285 | 1925 | 72 | 6423 | 5193 | 112 | 9440 | 8650 | 152 | 12399 | 12198 |
| 33 | 3366 | 2003 | 73 | 6499 | 5278 | 113 | 9515 | 8738 | 153 | 12473 | 12288 |
| 34 | 3447 | 2080 | 74 | 6576 | 5363 | 114 | 9589 | 8826 | 154 | 12546 | 12377 |
| 35 | 3528 | 2158 | 75 | 6652 | 5448 | 115 | 9664 | 8914 | 155 | 12620 | 12467 |
| 36 | 3609 | 2237 | 76 | 6728 | 5533 | 116 | 9738 | 9002 | 156 | 12693 | 12556 |
| 37 | 3689 | 2315 | 77 | 6805 | 5618 | 117 | 9813 | 9090 | 157 | 12767 | 12646 |
| 38 | 3769 | 2394 | 78 | 6881 | 5704 | 118 | 9887 | 9178 | 158 | 12840 | 12736 |
| 39 | 3850 | 2473 | 79 | 6957 | 5789 | 119 | 9962 | 9266 | 159 | 12913 | 12826 |
|  |  |  |  |  |  |  |  |  | 160 | NA | 12915 |
|  |  |  |  |  |  |  |  |  | Test limit = 1.2352E-2 |  |  |

Table G.4.4-2: pass fail limits for ER = 0.001

| ne | nsp | nsf | ne | nsp | nsf | ne | nsp | nsf | ne | nsp | nsf |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 3463 | NA | 41 | 40174 | 26265 | 82 | 71961 | 60368 | 123 | 102723 | 96075 |
| 1 | 4874 | 4 | 42 | 40971 | 27063 | 83 | 72720 | 61225 | 124 | 103465 | 96958 |
| 2 | 6096 | 99 | 43 | 41766 | 27863 | 84 | 73479 | 62083 | 125 | 104208 | 97842 |
| 3 | 7226 | 329 | 44 | 42559 | 28666 | 85 | 74237 | 62941 | 126 | 104949 | 98726 |
| 4 | 8298 | 658 | 45 | 43352 | 29471 | 86 | 74995 | 63801 | 127 | 105691 | 99610 |
| 5 | 9330 | 1059 | 46 | 44142 | 30279 | 87 | 75752 | 64661 | 128 | 106432 | 100495 |
| 6 | 10332 | 1513 | 47 | 44932 | 31088 | 88 | 76509 | 65522 | 129 | 107173 | 101380 |
| 7 | 11310 | 2009 | 48 | 45720 | 31899 | 89 | 77265 | 66383 | 130 | 107914 | 102266 |
| 8 | 12269 | 2539 | 49 | 46507 | 32713 | 90 | 78020 | 67246 | 131 | 108655 | 103152 |
| 9 | 13212 | 3096 | 50 | 47293 | 33528 | 91 | 78776 | 68109 | 132 | 109395 | 104039 |
| 10 | 14141 | 3677 | 51 | 48078 | 34345 | 92 | 79530 | 68973 | 133 | 110135 | 104926 |
| 11 | 15058 | 4278 | 52 | 48861 | 35164 | 93 | 80285 | 69838 | 134 | 110875 | 105813 |
| 12 | 15965 | 4896 | 53 | 49644 | 35984 | 94 | 81038 | 70704 | 135 | 111614 | 106701 |
| 13 | 16863 | 5530 | 54 | 50425 | 36807 | 95 | 81792 | 71570 | 136 | 112353 | 107589 |
| 14 | 17753 | 6177 | 55 | 51205 | 37631 | 96 | 82544 | 72437 | 137 | 113092 | 108478 |
| 15 | 18635 | 6836 | 56 | 51985 | 38456 | 97 | 83297 | 73305 | 138 | 113830 | 109367 |
| 16 | 19511 | 7507 | 57 | 52763 | 39283 | 98 | 84049 | 74173 | 139 | 114569 | 110257 |
| 17 | 20380 | 8188 | 58 | 53541 | 40112 | 99 | 84800 | 75042 | 140 | 115307 | 111146 |
| 18 | 21244 | 8878 | 59 | 54317 | 40942 | 100 | 85551 | 75911 | 141 | 116045 | 112037 |
| 19 | 22103 | 9576 | 60 | 55092 | 41773 | 101 | 86302 | 76782 | 142 | 116782 | 112927 |
| 20 | 22957 | 10282 | 61 | 55867 | 42606 | 102 | 87052 | 77653 | 143 | 117520 | 113818 |
| 21 | 23806 | 10995 | 62 | 56641 | 43440 | 103 | 87802 | 78524 | 144 | 118257 | 114710 |
| 22 | 24652 | 11715 | 63 | 57414 | 44276 | 104 | 88552 | 79396 | 145 | 118994 | 115602 |
| 23 | 25493 | 12441 | 64 | 58186 | 45113 | 105 | 89301 | 80269 | 146 | 119730 | 116494 |
| 24 | 26331 | 13173 | 65 | 58957 | 45951 | 106 | 90050 | 81143 | 147 | 120466 | 117386 |
| 25 | 27166 | 13911 | 66 | 59728 | 46790 | 107 | 90798 | 82017 | 148 | 121203 | 118279 |
| 26 | 27997 | 14654 | 67 | 60497 | 47631 | 108 | 91546 | 82891 | 149 | 121939 | 119173 |
| 27 | 28826 | 15401 | 68 | 61266 | 48472 | 109 | 92293 | 83766 | 150 | 122674 | 120066 |
| 28 | 29651 | 16154 | 69 | 62035 | 49315 | 110 | 93041 | 84642 | 151 | 123410 | 120960 |
| 29 | 30474 | 16910 | 70 | 62802 | 50159 | 111 | 93787 | 85518 | 152 | 124145 | 121855 |
| 30 | 31294 | 17671 | 71 | 63569 | 51004 | 112 | 94534 | 86395 | 153 | 124880 | 122749 |
| 31 | 32111 | 18436 | 72 | 64335 | 51851 | 113 | 95280 | 87273 | 154 | 125615 | 123644 |
| 32 | 32927 | 19204 | 73 | 65100 | 52698 | 114 | 96026 | 88151 | 155 | 126349 | 124540 |
| 33 | 33740 | 19976 | 74 | 65865 | 53546 | 115 | 96771 | 89029 | 156 | 127083 | 125435 |
| 34 | 34551 | 20752 | 75 | 66629 | 54396 | 116 | 97516 | 89908 | 157 | 127818 | 126332 |
| 35 | 35360 | 21531 | 76 | 67393 | 55246 | 117 | 98261 | 90788 | 158 | 128551 | 127228 |
| 36 | 36166 | 22312 | 77 | 68156 | 56097 | 118 | 99005 | 91668 | 159 | 129285 | 128125 |
| 37 | 36971 | 23097 | 78 | 68918 | 56950 | 119 | 99750 | 92548 | 160 | 130019 | 129022 |
| 38 | 37775 | 23885 | 79 | 69679 | 57803 | 120 | 100493 | 93429 | 161 | 130752 | 129919 |
| 39 | 38576 | 24676 | 80 | 70440 | 58657 | 121 | 101237 | 94311 | 162 | NA | 130817 |
| 40 | 39376 | 25469 | 81 | 71201 | 59512 | 122 | 101980 | 95193 | Test limit = 1.2345E-3 |  |  |

NOTE 1:	The first column is the number of errors (ne = number of misdetections)

NOTE 2:	The second column is the number of samples for the pass limit (nsp, ns=Number of Samples= number misdetections + number of detections)

NOTE 3:	The third column is the number of samples for the fail limit (nsf)

NOTE 4:	The test limit at the end of the table is applicable, when the minimum test time in clause 3.5 governs the test. Pass the test for ER ≤ Test limit, otherwise fail.

## G.4.5	Pass fail decision rules

G.2.5 applies

NOTE:	For ER=0.01 an ideal DUT passes after 344 samples. The maximum test time is 12913 samples. For ER=0.001 an ideal DUT passes after 3463 samples. The maximum test time is 130752 samples.

For LAA the following also apply:

The pass fail decision is done individually for each CC. The pass fail decision for one component in the test vector is as follows: pass if all CCs or SCC only according to the test cases pass, otherwise fail.

It is allowed to apply more samples to the DUT, common for all CCs, (e.g. up to an integer number of frames). Use the ratio for the pass fail decision.

## G.4.6	Minimum Test time

Table G.4.6-1: Minimum Test time for Demodulation of PCFICH/PDCCH

| Test No | Demod. Scenario | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) |  | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | FDD | TDD | FDD | TDD |
| 1 | [8.1] | R.15 (10 MHz, 8CCE, full, QPSK 1/3) (1x2 Low) ETU70 | 200 000 | 222 222 | 400 000 | 224 000 | 401 000 |
| 1 | [8.2] | R.16 (1.4MHz, 2CCE, full, QPSK 1/3) (2x2 Low) EPA5 | 200 000 | 222 222 | 400 000 | 224 000 | 401 000 |
| 1 Rel-9 … | [ ] | R.16_1 (10MHz, 4CCE, full, QPSK 1/3) (2x2 Low) EVA70 | 200 000 | 222 222 | 400 000 | 224 000 | 401 000 |
| 1 | [8.3] | R.17 (10MHz, 4CCE, full, QPSK 1/3) (4x2 Medium) EVA5 | 200 000 | 222 222 | 400 000 | 224 000 | 401 000 |
| 1 Rel-9 … | [ ] | R.17_1 (5MHz, 2CCE, full, QPSK 1/3) (4x2 Medium) EPA5 | 200 000 | 222 222 | 400 000 | 224 000 | 401 000 |
| Note: Simulation method to derive MNAS is based on finite test time and its effect on test system uncertainty specified in clause F.1.4. |  |  |  |  |  |  |  |

Table G.4.6-1a: Minimum Test time for Demodulation of PCFICH/PDCCH for eICIC

| Test No | Demod. Scenario | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) |  | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | FDD | TDD | FDD | TDD |
| 1 | [ ] | R.15-1 (10 MHz, 8CCE, full, QPSK 1/3) (2x2 Low) EVA5 | 200 000 (MNAS is not simulated, but estimated based on similar scenario in Table G.4.6-1 Test1) | 2 000 000 | 2 000 000 | 2 001 000 | 2 001 000 |
| Note: Simulation method to derive MNAS is based on finite test time and its effect on test system uncertainty specified in clause F.1.4. |  |  |  |  |  |  |  |

Table G.4.6-1b: Minimum Test time for Demodulation of PCFICH/PDCCH for feICIC

| Test No | Demod. Scenario | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) |  | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | FDD | TDD | FDD | TDD |
| 1 | [ ] | R.15-2 (10 MHz, 8CCE, full, QPSK 1/3) (2x2 Low) EVA5 | 200 000 (MNAS is not simulated, but estimated based on similar scenario in Table G.4.6-1 Test1) | 2 000 000 | 2 000 000 | 2 001 000 | 2 001 000 |
| Note: Simulation method to derive MNAS is based on finite test time and its effect on test system uncertainty specified in clause F.1.4. |  |  |  |  |  |  |  |

Table G.4.6-2: Minimum Test time for Demodulation of PHICH

| Test No | Demod. scenario | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) |  | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | FDD | TDD | FDD | TDD |
| 1 | [9.1] | R.18 (10 MHz, full, QPSK 1/3) (1x2 Low) ETU70 | 200 000 | 200 000 | 500 000 | 200 000 | 500 000 |
| 2 | [9.4] | R.24 (10MHz, full, 16QAM ½) (1x2 Low) ETU70 | 200 000 | 200 000 | 500 000 | 200 000 | 500 000 |
| 1 | [9.2] | R.19 (1.4MHz, full, 64QAM 3/4) (2x2 Low) EPA5 | 200 000 | 200 000 | 500 000 | 200 000 | 500 000 |
| 1 Rel-9… | [ ] | R.19_1 (10MHz, full, 64QAM 3/4) (2x2 Low) EVA70 | 200 000 | 200 000 | 500 000 | 200 000 | 500 000 |
| 1A |  | R.19-1 (5MHz, full, 64QAM 3/4) (2x2 Low) EVA70 | 200 000 | 200 000 | 500 000 | 200 000 | 500 000 |
| 1 | [9.3] | R.20 (10MHz, 1PRB, 16QAM ½) (4x2 Medium) EVA5 | 200 000 | 200 000 | 500 000 | 200 000 | 500 000 |
| 1 Rel-9… | [ ] | R.20_1 (5MHz, 1PRB, 16QAM ½) (4x2 Medium) EPA5 | 200 000 | 200 000 | 500 000 | 200 000 | 500 000 |
| Note: Simulation method to derive MNAS is based on finite test time and its effect on test system uncertainty specified in clause F.1.4. |  |  |  |  |  |  |  |

Table G.4.6-2a: Minimum Test time for Demodulation of PHICH for eICIC

| Test No | Demod. scenario | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) |  | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | FDD | TDD | FDD | TDD |
| 1 | [ ] | R.19_1 (10MHz, full, 64QAM 3/4) (2x2 Low) EPA5 | 200 000 (MNAS is not simulated, but estimated based on similar scenario in Table G.4.6-2 Test1) | 1 600 000 | 2 000 000 | 1 600 000 | 2 000 000 |
| Note: Simulation method to derive MNAS is based on finite test time and its effect on test system uncertainty specified in clause F.1.4. |  |  |  |  |  |  |  |

Table G.4.6-2b: Minimum Test time for Demodulation of PHICH for feICIC

| Test No | Demod. scenario | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) |  | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | FDD | TDD | FDD | TDD |
| 1 | [ ] | R.19_1 (10MHz, full, 64QAM 3/4) (2x2 Low) EPA5 | 200 000 (MNAS is not simulated, but estimated based on similar scenario in Table G.4.6-2 Test1) | 1 600 000 | 2 000 000 | 1 600 000 | 2 000 000 |
| Note: Simulation method to derive MNAS is based on finite test time and its effect on test system uncertainty specified in clause F.1.4. |  |  |  |  |  |  |  |

Table G.4.6-3: Minimum Test time for PHICH for UE Category 0 and UE Category 1bis

| Clause 8.9.2.1.1/8.9.2.1.1_1/8.9.2.2.1/8.9.2.2.1_1 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) |  | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | HD-FDD | TDD | HD-FDD | TDD |
| 1 | R.19 (10MHz, 1PRB, PBSK, 1/2) (2x1 Low) EPA5 | 46 524 (MNAS is not simulated, but estimated based on similar scenarios in Table G.3.5-1 Test11) with HD-FDD pattern | 124 064 | 93 048 | 126 000 | 95 000 |

Table G.4.6-4: Minimum Test time for Demodulation of PCFICH/PDCCH for 4 Rx antenna ports

| Test No | Demod. Scenario | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) |  | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | FDD | TDD | FDD | TDD |
| 8.10.2.1.1, 8.10.2.2.1: Test 1 | [ ] | R.15 (10 MHz, 8CCE, full, QPSK 1/3) (1x4 Low) ETU70 | 200 000 (MNAS is not simulated, but estimated based on similar scenario in Table G.4.6-1 Test 1) | 222 222 | 400 000 | 224 000 | 401 000 |
| 8.10.2.1.2, 8.10.2.2.2: Test 1 | [ ] | R.16_1 (10MHz, 4CCE, full, QPSK 1/3) (2x4 Low) EVA70 | 200 000 (MNAS is not simulated, but estimated based on similar scenario in Table G.4.6-1 Test 1) | 222 222 | 400 000 | 224 000 | 401 000 |
| 8.10.2.1.3, 8.10.2.2.3: Test 1 | [ ] | R.17_1 (5MHz, 2CCE, full, QPSK 1/3) (4x4 Medium A Xpol) EPA5 | 200 000 (MNAS is not simulated, but estimated based on similar scenario in Table G.4.6-1 Test 1) | 222 222 | 400 000 | 224 000 | 401 000 |

Table G.4.6-4.0: Minimum Test time for Demodulation of LAA PCFICH/PDCCH

| Test No | Demodulation scenario (info only) | MNAS (Note 1) | MNS (Calculation) |  | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD | FDD | TDD |
| 8.4.3.1.1 Test 1 for PCC | PCC: R.16_1 FDD (10 MHz, 4CCE, full, QPSK 1/3) (2x2 Low) EVA5 | 200 000 | 222 222 | N/A | 224 000 | N/A |
| 8.4.3.1.1 Test 1-4 for SCC | SCC: R.3 FS3 (20 MHz, 4CCE, full, QPSK 1/3) (2x2 Low) EVA5 | 200 000 | N/A | 476 190 (Note 2) | N/A | 478 000 |
| Note 1: MNAS is not simulated, but estimated based on similar scenario in Table G.4.6-1 Test1. Note 2: MNS for SCC is not deterministic due to the statistical burst transmission model in clause B.3.8. The average MNS is presented in this table. This is calculated based on an average burst length of 5.25 ms which in average takes 6.25 ms assuming UE not capable of partial subframes, and a simulated probability of LBT success (p) equal to 0.5 (ratio 6.25/5.25/0.5). |  |  |  |  |  |  |

Table G.4.6-4a: Minimum Test time for Demodulation of PHICH for 4 Rx antenna ports

| Test No | Demod. Scenario | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) |  | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | FDD | TDD | FDD | TDD |
| 8.10.3.1.1, 8.10.3.2.1: Test 1 | [ ] | R.18 (10 MHz, full, QPSK 1/3) (1x4 Low) ETU70 | 200 000 (MNAS is not simulated, but estimated based on similar scenario in Table G.4.6-2 Test 1) | 200 000 | 500 000 | 200 000 | 500 000 |
| 8.10.3.1.2, 8.10.3.2.2: Test 1 | [ ] | R.19_1 (10MHz, full, 64QAM 3/4) (2x4 Low) EVA70 | 200 000 (MNAS is not simulated, but estimated based on similar scenario in Table G.4.6-2 Test 1) | 200 000 | 500 000 | 200 000 | 500 000 |
| 8.10.3.1.3, 8.10.3.2.3: Test 1 | [ ] | R.20_1 (5MHz, 1PRB, 16QAM ½) (4x4 Medium correlation A, Cross polarized) EPA5 | 200 000 (MNAS is not simulated, but estimated based on similar scenario in Table G.4.6-2 Test 1) | 200 000 | 500 000 | 200 000 | 500 000 |

Table G.4.6-5: Minimum Test time for MPDCCH for UE Category M1 for FDD & HD-FDD

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) | MNSF (Min No Sub Frames, mandatory) |
| --- | --- | --- | --- | --- |
|  |  |  | FDD & HD-FDD | FDD & HD-FDD |
| 8.11.2.1.1: Test 1 | R.82 FDD (10MHz, 4 PRB, QPSK, 1/2) (2x1 Low) EPA5 | 97 280 | 291 840 | 293 000 |
| 8.11.2.1.2: Test 1 | R.83 FDD (10MHz, 2+4 PRB, QPSK, 1/2) (2x1 Low) ETU1 | 194 560 | 583 680 | 585 000 |

Table G.4.6-5a: Minimum Test time for MPDCCH for UE Category M1 for TDD

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) | MNSF (Min No Sub Frames, mandatory) |
| --- | --- | --- | --- | --- |
|  |  |  | TDD | TDD |
| 8.11.2.2.1: Test 1 | R.82 TDD (10MHz, 4 PRB, QPSK, 1/2) (2x1 Low) EPA5 | 48 640 | 486 400 | 488 000 |
| 8.11.2.2.2: Test 1 | R.83 TDD (10MHz, 2+4 PRB, QPSK, 1/2) (2x1 Low) ETU1 | 97 280 | 972 800 | 974 000 |

Table G.4.6-6: Minimum Test time for Demodulation of LAA PCFICH/PDCCH

| Test No | Demodulation scenario (info only) | MNAS (Note 1) | MNS (Calculation) |  | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD | FDD | TDD |
| 8.4.3.1.2 Test 1 for PCC | PCC: R.16_1 TDD (10 MHz, 4CCE, full, QPSK 1/3) (2x2 Low) EVA5 | 200 000 | N/A | 400 000 | N/A | 401 000 |
| 8.4.3.1.2 Test 1-4 for SCC | SCC: R.4 FS3 (20 MHz, 4CCE, full, QPSK 1/3) (2x2 Low) EVA5 | 200 000 | N/A | 476 190 (Note 2) | N/A | 478 000 |
| Note 1: MNAS is not simulated, but estimated based on similar scenario in Table G.4.6-1 Test1. Note 2: MNS for SCC is not deterministic due to the statistical burst transmission model in clause B.3.8. The average MNS is presented in this table. This is calculated based on an average burst length of 5.25 ms which in average takes 6.25 ms assuming UE not capable of partial subframes, and a simulated probability of LBT success (p) equal to 0.5 (ratio 6.25/5.25/0.5). |  |  |  |  |  |  |

Table G.4.6-7: Minimum Test time for NPDCCH for UE Category NB1

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) | MNSF (Min No Sub Frames, mandatory) |
| --- | --- | --- | --- | --- |
|  |  |  | HD-FDD | HD-FDD |
| 8.12.2.1.1: Test 1 | R.NB.3 FDD (200kHz, QPSK,1/2) (1x1) EPA5 | 36166 | 15430827 | 15431000 |
| 8.12.2.1.1: Test 2 | R.NB.3 FDD (200kHz, QPSK,1/2) (1x1) ETU1 | 36166 | 55550976 | 55551000 |

Table G.4.6-8: Minimum Test time for NPDCCH Transmit diversity performance for UE Category NB1

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) | MNSF (Min No Sub Frames, mandatory) |
| --- | --- | --- | --- | --- |
|  |  |  | HD-FDD | HD-FDD |
| 8.12.2.1.2: Test 1 | R.NB.4 FDD (200kHz, QPSK,1/2) (2x1) EPA5 | 36166 | 4629248 | 4631000 |
| 8.12.2.1.2: Test 2 | R.NB.4 FDD (200kHz, QPSK,1/2) (2x1) ETU1 | 36166 | 27775488 | 27777000 |

## G.4.7	Test conditions for receiver performance tests

Table G.4.7: Test conditions for receiver performance tests

| Test | Statistical independence | Number of components in the test vector, as specified in the test requirements and initial conditions of the applicable test | Over all Pass/Fail condition Restrictions and extensions see Table G.3.6-1 |
| --- | --- | --- | --- |
| 8.4.1.1 FDD PCFICH/PDCCH Single-antenna Port Performance | A misdetection is an independent event | 1 | NA |
| 8.4.1.2 FDD PCFICH/PDCCH Transmit Diversity Performance | A misdetection is an independent event | 2 | To pass 8.4.1.2 each component in the test vector must pass |
| 8.4.2.1 TDD PCFICH/PDCCH Single-antenna Port Performance | A misdetection is an independent event | 1 | NA |
| 8.4.2.2 TDD PCFICH/PDCCH Transmit Diversity Performance | A misdetection is an independent event | 2 | To pass 8.4.2.2 each component in the test vector must pass |
| 8.5.1.1 FDD PHICH Single-antenna Port Performance | A misdetection is an independent event | 2 | To pass 8.5.1.1 each component in the test vector must pass |
| 8.5.1.2 FDD PHICH Transmit Diversity Performance | A misdetection is an independent event | 2 | To pass 8.5.1.2 each component in the test vector must pass |
| 8.5.2.1 TDD PHICH Single-antenna Port Performance | A misdetection is an independent event | 2 | To pass 8.5.2.1 each component in the test vector must pass |
| 8.5.2.2 TDD PHICH Transmit Diversity Performance | A misdetection is an independent event | 2 | To pass 8.5.2.2 each component in the test vector must pass |

# G.5	Measuring throughput ratio

## G.5.1	General

Annex G.5 is applicable for clauses 9.3, 9.4, 9.5 and 9.7. Common to those clauses is, that a throughput ratio γ of the form  γ=tNumeratortDenominator is measured. These clauses are tested exclusively with “slow” multipath fading profiles. Hence the test time is governed by test time due to fading, and number of samples due to statistical significance is not applicable.

The test requirement in clause 9.4 is a ratio of 2 throughput tests γ. In either numerator or denominator (depending on test case) a target throughput is desired, which is established by an approach resulting in the throughput and the reference SNR. This SNR is then reused when measuring the throughput of the other factor of the formula. The formulas for calculation of γ are defined directly under sections 9.4.

The test requirements in clauses 9.3 and 9.5 are a ratio of 2 throughput tests γ. , where numerator and denominator are ordinary throughput tests. The formulas for calculation of γ are defined in sections 9.3 and 9.5 respectively

## G.5.2	Establishing SNR

Adjust SNR such that the measured throughput is within 2% of target value (60, 70 or 90% depending on test case). The approach, leading to target throughput and reference SNR is not specified.

The resulting SNR is the reference SNR to use when measuring throughput in the other factor (numerator or denominator) of γ.

To achieve statistical significance the final throughput measurement must be done with MNS samples, given table G.5.4-1

## G.5.3	Measuring T-put

To achieve statistical significance the final throughput measurement must be done with MNS samples, given in table.G.5.4 -1. Number of samples due to statistical significance is not applicable.

For measuring tsubband, twideband, treported and tfix, the SS collects ACK, NACK and statDTX from the UE and records the time, elapsed from the beginning of the test. The payload size, received by the UE and acknowledged towards the SS, may vary within a test versus time (e.g. due to subband changes upon a UE report) Throughput is calculated in the SS by summing up the payload, associated to each ACK, from the start of the test and dividing the accumulated payload in kilobits by the time in seconds, elapsed from the beginning of the test. This is similar but not same as in G.2.2. (Main difference in bullet d, where the payload size is constant).

For measuring tmedian, tue, trnd, tue,follow1,follow2 and tue,rnd1,rnd2, the SS collects ACK, NACK and statDTX from the UE and records the time, elapsed from the beginning of the test. The payload size, received by the UE and acknowledged towards the SS, is constant. Throughput can be calculated in the SS by multiplying the payload size with the number of ACKs and dividing the accumulated payload in kilobits by the time in seconds, elapsed from the beginning of the test, being associated to the following ratio: ACK/ (ACK+NACK + DTX).

## G.5.4	Number of samples for throughput ratios

TT for γ and MNS are based on theoretical estimations.

Table G.5.4-1: Test time for testing throughput ratios

| Test | Demodulation scenario: RMC (Bandwidth, allocated RBs, modulation, coding) [Antenna configuration, correlation] Propagation condition, Doppler | Γ | Minimum Number of Subframes (MNS) (MNS = active and inactive Subframes, more details in Annex G.3.5 and table G.3.5-1) |  | Γ including TT | BLER |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |  |  |
| 9.3.1.1.1 9.3.1.1.2 | (10 MHz, 6, variable modulation and coding) [1x2, full] Special propagation according to clause B.2.4, 5Hz | 1.1 | 100000 For denominator- and numerator - measurement each | 170000 For denominator- and numerator - measurement each | γ =1.09 | BLER=0.05, no TT No of samples: subset of ACKs and NACKs in the MNS for throughput. |
| 9.3.1.2.1_D | (10 MHz, 6, variable modulation and coding) [2x2, full] Special propagation according to clause B.2.4, 5Hz | 1.1 | 100000 For denominator- and numerator - measurement each | NA | γ =1.09 | BLER=0.05, no TT No of samples: subset of ACKs and NACKs in the MNS for throughput. |
| 9.3.1.2.2_D | (10 MHz, 6, variable modulation and coding) [2x2, full] Special propagation according to clause B.2.4, 5Hz | 1.1 | NA | 170000 For denominator- and numerator- measurement each | γ =1.09 | BLER=0.05, no TT No of samples: subset of ACKs and NACKs in the MNS for throughput. |
| 9.3.1.3.1_E | (10 MHz, 6, variable modulation and coding) [1x2, full] Special propagation according to clause B.2.4, 5Hz | 1.1 | 200000 For denominator- and numerator - measurement each | NA | γ =1.09 | BLER=0.01, no TT No of samples: subset of ACKs and NACKs in the MNS for throughput. |
| 9.3.1.3.2_E | (10 MHz, 6, variable modulation and coding) [1x2, full] Special propagation according to clause B.2.4, 5Hz | 1.1 | NA | 680000 For denominator- and numerator - measurement each | γ =1.09 | BLER=0.01, no TT No of samples: subset of ACKs and NACKs in the MNS for throughput. |
| 9.3.2.1.1 9.3.2.1.2 | (10 MHz or 5MHz according to test, full, variable modulation and coding) [1x2, high] EPA5 | 1.05 | 100000 For denominator- and numerator - measurement each | 170000 For denominator- and numerator - measurement each | γ =1.04 | BLER=0.02, no TT No of samples for FDD: subset of ACKs and NACKs in the MNS for throughput. No of samples for TDD: subset of filtered ACKs and NACKs in the MNS for throughput. |
| 9.3.2.1.1_1 9.3.2.1.2_1 | (10 MHz or 5MHz according to test, partial, variable modulation and coding) [1x2, high] EPA5 | 1.05 | 100000 For denominator- and numerator - measurement each | 170000 For denominator- and numerator - measurement each | γ =1.04 | BLER=0.02, no TT No of samples for FDD: subset of ACKs and NACKs in the MNS for throughput. No of samples for TDD: subset of filtered ACKs and NACKs in the MNS for throughput. |
| 9.3.2.2.1_D | (10 MHz, full, variable modulation and coding) [4x2, high] EPA5 | 1.05 | 100000 For denominator- and numerator - measurement each | NA | γ =1.04 | BLER=0.02, no TT No of samples for FDD: subset of ACKs and NACKs in the MNS for throughput. |
| 9.3.2.2.2_D | (10 MHz, full, variable modulation and coding) [8x2, high] EPA5 | 1.05 | NA | 170000 For denominator- and numerator - measurement each | γ =1.04 | BLER=0.02, no TT No of samples for TDD: subset of filtered ACKs and NACKs in the MNS for throughput. |
| 9.3.3.1.1 9.3.3.1.2 | 10 MHz, full(however unequal SNR), variable modulation and coding) [1x2, full] Special: propagation according to clause B.2.4, 5Hz | 1.6 | 100000 For denominator- and numerator - measurement each | 170000 For denominator- and numerator - measurement each | 1.50 |  |
| 9.3.4.1.1 9.3.4.1.2 | Same as 9.3.3 | 1.2 | 100000 | 170000 | 1.19 |  |
| 9.3.4.2.1 9.3.4.2.2 | Same as 9.3.3 | 1.15 | 100000 | 170000 | 1.14 |  |
| 9.3.5.1.1 9.3.5.1.2 | variable modulation and coding, 1x2 Low, EPA5 | 1.8 | 100000 | 170000 | 1.79 |  |
| 9.3.5.2.1 9.3.5.2.2 | variable modulation and coding, 2x2 Low, EPA5 | 1.8 | 100000 | 170000 | 1.79 |  |
| 9.3.7.1 9.3.7.2 | Test1: (10 MHz, full, variable modulation and coding) [4x2, low]EVA5 Test2: (10 MHz, full, variable modulation and coding) [4x2, High XP] EVA5 | Test1: 1.05 Test2: 1.15 | 100000 | 170000 | Test1: 1.04 Test2: 1.14 |  |
| 9.3.8.1.1 9.3.8.1.2 | variable modulation and coding, 2x2 Low, EPA5 | 0.925 | 100000 | 170000 | 0.915 |  |
| 9.3.8.2.1 9.3.8.2.2 | variable modulation and coding, 2x2 Low, EPA5 | 0.925 | 100000 | 170000 | 0.915 |  |
| 9.3.8.3.1 9.3.8.3.2 | variable modulation and coding, 2x2 Low, EPA5 | 0.925 | 100000 | 170000 | 0.915 |  |
| 9.4.1.1.1 9.4.1.1.2 | R.10 (10 MHz, full, QPSK, 1/3) [2x2, Low] EVA5 | 1.1 | 100000 | 170000 | γ =1.09 |  |
| 9.4.1.2.1 9.4.1.2.2 | R.14-1 (10 MHz, partial, 16QAM,1/2) [4x2, Low] EVA5 | 1.2 | 100000 | 170000 | 1.19 |  |
| 9.4.2.1.1 9.4.2.1.2 | R.30 (20 MHz, full, 16QAM, 1/2) [2x2, Low] EPA5 | 1.2 | 100000 | 170000 | γ =1.19 |  |
| 9.4.2.1.1_1 9.4.2.1.2_1 | R.11-3 or R.11 (10 MHz, partial or full, 16QAM, 1/2) [2x2, Low] EPA5 | 1.2 | 100000 | 170000 | γ =1.19 |  |
| 9.4.2.2.1 | R.14-2 FDD (10MHz, 3, 16 QAM, ½) [4x2, low] EVA5 | 1.2 | 100000 | NA | 1.19 |  |
| 9.4.2.2.2 | R.14-2 TDD (10MHz, 3, 16QAM, ½) [4x2, low] EVA5 | 1.15 | NA | 170000 | 1.14 |  |
| 9.4.1.3.1_D | R.44 (10 MHz, partial, QPSK, 1/3) [4x2, Low] EPA5 | 1.2 | 100000 | NA | γ =1.19 |  |
| 9.4.1.3.3 | R.77 (10 MHz, 50, 64QAM, 1/3) [High 2D XP 12(2,3,2) x 2] EPA5 | 2.5 | 100000 | NA | 2.49 |  |
| 9.4.1.3.4 | R.77 (10 MHz, 50, 64QAM, 1/3) [High 2D XP 12(2,3,2) x 2] EPA5 | 2.5 | NA | 17000 | 2.49 |  |
| 9.4.1.3.2_D | R.45-1 or R.45 (10 MHz, partial or full, 16QAM, 1/2) [8x2, High] EVA5 | 3 | NA | 170000 | γ =2.99 |  |
| 9.4.1.4.1 9.4.1.4.2 | R.60 (10MHz, full, QPSK, 1/2) [4x2, High XP] EVA5 | 1.8 | 100000 | 170000 | γ =1.79 |  |
| 9.4.1.4.3 | R.45 (10MHz, full, QPSK, 1/2) [ULA Low 4 x 2] EPA5 | 1.1 | 100000 | NA | γ =1.09 |  |
| 9.4.1.4.4 | R.45 (10MHz, full, QPSK, 1/2) [ULA Low 4 x 2] EPA5 | 1.2 | NA | 170000 | γ =1.19 |  |
| 9.4.2.3.1_D | R.45-1 or R.45 (10 MHz, partial or full, 16QAM, 1/2) [4x2, Low] EVA5 | 1.3 | 100000 | NA | γ =1.29 |  |
| 9.4.2.3.2_D | R.45-1 or R.45 (10 MHz, partial or full, 16QAM, 1/2) [8x2, High] EVA5 | 3.5 | NA | 170000 | γ =3.49 |  |
| 9.4.2.3.3 | R.45-1 or R.45 (10 MHz, partial or full, 16QAM, 1/2) [4x2, High XP] EVA5 | 1.2 | 100000 | NA | γ =1. 19 |  |
| 9.4.2.3.4 | R.61-1 or R.61(10MHz, full, 16QAM, 1/2) [4x2, High XP] EVA5 | 1.2 | NA | 170000 | γ =1. 19 |  |
| 9.4.2.3.5 | R.45-1 or R.45 (10 MHz, partial or full, 16QAM, 1/2) [4x2, High XP] EVA5 | 2.5 | 100000 | NA | γ =2.49 |  |
| 9.4.2.3.6 | R.45-1 or R.45 (10 MHz, partial or full, 16QAM, 1/2) [4x2, High XP] EVA5 | 2.5 | NA | 170000 | γ =2.49 |  |
| 9.5.1.1 9.5.1.2 | (10MHz, full, variable modulation and coding) [2x2, low or high according to test] EPA5 | Test21=1.05 Test12=1 Test32=1.1 | 100000 | 170000 | Test21=1.04 Test12=0.99 Test32=1.09 |  |
| 9.5.1.1_1 9.5.1.2_1 | (10MHz, full, variable modulation and coding) [2x2, low or high according to test] EPA5 | Test21=1.05 Test12=1 Test3: 1=0.90 2=1.10 | 100000 | 170000 | Test21=1.04 Test12=0.99 Test3: 1=0.89 2=1.09 |  |
| 9.5.1.1_2 9.5.1.2_2 | (10MHz, full, variable modulation and coding) [2x2, low or high according to test] EPA5 | Test21=1.05 Test12=1 Test3: 1=0.90 | 100000 | 170000 | Test21=1.04 Test12=0.99 Test3: 1=0.89 |  |
| 9.5.2.1_D 9.5.2.2_D | (10MHz, full, variable modulation and coding) [2x2, low or high according to test] EPA5 | Test21=1.05 Test31=0.9 Test12=1.00 | 100000 | 170000 | Test21=1.04 Test31=0.89 Test12=0.99 |  |
| 9.5.3.1_C.1 9.5.3.2_C.1 | (10MHz, full, variable modulation and coding) [2x2, low according to test] EPA5 | Test11=0.9 Test21=1.05 | 800000 | 680000 | Test11=0.89 Test21=1.04 |  |
| 9.5.4.1_E 9.5.4.2_E | (10MHz, full, variable modulation and coding) [2x2, low or high according to test] EPA5 | Test21=1.05 Test31=0.9 Test12=1.00 | 800000 | 680000 | Test21=1.04 Test31=0.89 Test12=0.99 |  |
| 9.5.5.1_F.2 9.5.5.2_F.2 | (10MHz, full, variable modulation and coding) [2x2, low or high according to test] EPA5 | Test21=1.00 | 100000 | 170000 | Test21=0.99 |  |
| 9.7.2.1 9.7.2.2 | (10 MHz, 6, variable modulation and coding) [1x1, full] Special propagation according to clause B.2.4, 5Hz | 1.1 | 100000 | 170000 | γ =1.09 |  |
| 9.8.2.1 | (10 MHz, 6, variable modulation and coding) [1x1, full] Special propagation according to clause B.2.4, 1Hz | 1.30 | 800000 | NA | 1.29 |  |
| 9.9.2.1.1 9.9.2.1.2 | variable modulation and coding, 1x4 Low, EPA5 | 1.8 | 100000 | 170000 | 1.79 |  |
| 9.9.2.2.1 9.9.2.2.2 | variable modulation and coding, 2x4 Low, EPA5 | 1.8 | 100000 | 170000 | 1.79 |  |
| 9.9.3.1.1 | R.45-2 (10 MHz, full, 64QAM) [8x4, High] EVA5 | 2.5 | NA | 170000 | γ =2.49 |  |
| 9.9.4.1.1 9.9.4.1.2 | (10MHz, full, variable modulation and coding) [2x4, low or high for test 1, 2 and 3; 4x4, low for test 4 according to test] EPA5 | Test12=1 Test21=1.05 Test31=0.9 Test42=1.1 | 100000 | 170000 | Test12=0.99 Test21=1.04 Test31=0.89 Test42=1.09 |  |
| 9.9.4.2.1 9.9.4.2.2 | (10MHz, full, variable modulation and coding) [2x4, low or high for test 1, 2 and 3; 4x4, low for test 4 according to test] EPA5 | Test12=1 Test21=1.05 Test31=0.9 Test42=1.1 | 100000 | 170000 | Test12=0.99 Test21=1.04 Test31=0.89 Test42=1.09 |  |
| 9.10.1.1 |  | Test 1-1 1.2 Test 1-2 1.2 Test 1-3 1.3 Test 1-4 1.35 | 100000 | NA | Test 1-1 1.19 Test 1-2 1.19 Test 1-3 1.29 Test 1-4 1.34 |  |
| 9.10.1.2 |  | Test 1-1 1.2 Test 1-2 1.2 Test 1-3 1.3 Test 1-4 1.35 | NA | 170000 | Test 1-1 1.19 Test 1-2 1.19 Test 1-3 1.29 Test 1-4 1.34 |  |

# G.6	Statistical testing of MBMS Performance

## G.6.1	General

The system simulator sends MBMS packets to the UE under test. The number of packets, the SS sends, is predefined by the test time in G.6.4. The UE under tests demodulates the MBMS packets and counts the successfully received number of MBMS packets into the UE internal MBMS packet counter. The SS reads out the counter and issues a pass fail decision.

## G.6.2	Mapping of MBMS Packet ratio to BLER

The minimum requirements are designed in terms of BLER = 1%. 10 MBMS packets are included in one Transport block, constant throughout the test. The ratio of correct received MBMS packets (Mok) to transmitted packets (Mtot) is assumed to be equal to the ratio of correct received transport blocks to transmitted transport blocks. This assumption ignores the unlikely case that payload and CRC for a transport block are consistent but wrong.

## G.6.3	Design of the test

The minimum requirement for all MBMS tests is BLER = 1%. All MBMS tests are performed under a fading scenario: MBSFN channel model (Table B.2.6-1). It is obvious, that this fading scenario requires a minimum test time, greater than the time, required for statistical significance. In addition the test design with the MBMS packet counter in the UE suggest a fixed test time. Without simulation the test time is set to 200 000 active subframes (Transport blocks), leading to 333 333 subframes (333sec) for FDD and 400 000 subframes (400sec) for TDD. In order to avoid the fail of a good DUT due to statistical uncertainty the minimum requirement of BLER = 1% leads to a

Test limit = 1.2352 %. (refer Table G.4.4-1)

This means a DUT actually on the limit (1%) is measured and passed with a confidence level of greater than 95%.

## G.6.4	Test time for MBMS performance tests

Table G.6.4-1: Minimum Test time for MBMS

| Test No | Demodulation scenario (info only) | Min No of Sub Frames |  |
| --- | --- | --- | --- |
|  |  | FDD (6 of 10 subframes are active) | TDD (5 of 10 subframes are active) |
| 1 | R.37 (10 MHz, full, QPSK 1/3) (1x2 Low) MBSFN channel model | 333 333 | 400 000 |
| 2 | R.38 (10MHz, full, 16QAM 1/2) (1x2 Low) MBSFN channel model | 333 333 | 400 000 |
| 3 | R.39 (10 MHz, full, 64QAM 2/3) (1x2 Low) MBSFN channel model | 333 333 | 400 000 |
| 3 | R.39-1 (5 MHz, full, 64QAM 2/3) (1x2 Low) MBSFN channel model | 333 333 | 400 000 |
| 4 | R.40 (1.4 MHz, full, QPSK 1/3) (1x2 Low) MBSFN channel model | 333 333 | 400 000 |

# G.7	Theory to derive the numbers in Table G.2.4-1 (Informative)

Editor's note:	This clause of the Annex G is for information only and it described the background theory and information to derive the entries in the table G.2.4-1.

## G.7.1	Error Ratio (ER)

The Error Ratio (ER) is defined as the ratio of number of errors (ne) to all results, number of samples (ns).

(1-ER is the success ratio).

## G.7.2	Test Design

A statistical test is characterised by:

Test-time, Selectivity and Confidence level.

## G.7.3	Confidence level

The outcome of a statistical test is a decision. This decision may be correct or in-correct. The Confidence Level CL describes the probability that the decision is a correct one. The complement is the wrong decision probability (risk) D = 1-CL

## G.7.4	Introduction: Supplier Risk versus Customer Risk

There are two targets of decision:

(a)	A measurement on the pass-limit shows, that the DUT has the specified quality or is better with probability CL (CL e.g.95%) This shall lead to a "pass decision"

The pass-limit is on the good side of the specified DUT-quality. A more stringent CL (CL e.g.99%) shifts the pass-limit farer into the good direction. Given the quality of the DUTs is distributed, a greater CL passes less and better DUTs.

A measurement on the bad side of the pass-limit is simply “not pass” (undecided or artificial fail).

(aa)	Complementary:

A measurement on the fail-limit shows, that the DUT is worse than the specified quality with probability CL.

The fail-limit is on the bad side of the specified DUT-quality. A more stringent CL shifts the fail-limit farer into the bad direction. Given the quality of the DUTs is distributed, a greater CL fails less and worse DUTs.

A measurement on the good side of the fail-limit is simply “not fail”.

(b)	A DUT, known to have the specified quality, shall be measured and decided pass with probability CL. This leads to the test limit.

For CL e.g. 95%, the test limit is on the bad side of the specified DUT-quality. CL e.g.99% shifts the pass-limit farer into the bad direction. Given the DUT-quality is distributed, a greater CL passes more and worse DUTs.

(bb)	A DUT, known to be an (ε0) beyond the specified quality, shall be measured and decided fail with probability CL.

For CL e.g.95%, the test limit is on the good side of the specified DUT-quality.

NOTE 1:	The different sense for CL in (a), (aa) versus (b), (bb).

NOTE 2:	For constant CL in all 4 bullets (a) is equivalent to (bb) and (aa) is equivalent to (b).

## G.7.5	Supplier Risk versus Customer Risk

The table below summarizes the different targets of decision.

Table G.X.5-1: Equivalent statements

|  | Equivalent statements, using different cause-to-effect-directions, and assuming CL = constant >1/2 |  |
| --- | --- | --- |
| cause-to-effect-directions | Known measurement result estimation of the DUT’s quality | Known DUT’s quality estimation of the measurement’s outcome |
| Supplier Risk | A measurement on the pass-limit shows, that the DUT has the specified quality or is better (a) | A DUT, known to have an (ε0) beyond the specified DUT-quality, shall be measured and decided fail (bb) |
| Customer Risk | A measurement on the fail-limit shall shows, that the DUT is worse than the specified quality (aa) | A DUT, known to have the specified quality, shall be measured and decided pass (b) |

The shaded area shown the direct interpretation of Supplier Risk and Customer Risk.

The same statements can be based on other DUT-quality-definitions.

## G.7.6	Introduction: Standard test versus early decision concept

In standard statistical tests, a certain number of results (ns) is predefined in advance to the test. After ns results the number of bad results (ne) is counted and the error ratio (ER) is calculated by ne/ns.

Applying statistical theory, a decision limit can be designed, against which the calculated ER is compared to derive the decision. Such a limit is one decision point and is characterised by:

-	D: the wrong decision probability (a predefined parameter)

-	ns: the number of results (a fixed predefined parameter)

-	ne: the number of bad results (the limit based on just ns)

In the formula for the limit, D and ns can be understood as variable parameter and variable. However the standard test execution requires fixed ns and D. The property of such a test is: It discriminate between two states only, depending on the test design:

-	pass (with CL) / undecided           (undecided in the sense: finally undecided)

-	fail (with CL) / undecided            (undecided in the sense: finally undecided)

-	pass(with CL)  / fail (with CL)     (however against two limits).

In contrast to the standard statistical tests, the early decision concept predefines a set of (ne,ns) co-ordinates, representing the limit-curve for decision. After each result a preliminary ER is calculated and compared against the limit-curve. After each result one may make the decision or not (undecided for later decision). The parameters and variables in the limit-curve for the early decision concept have a similar but not equal meaning:

-	D: the wrong decision probability (a predefined parameter)

-	ns: the number of results (a variable parameter)

-	ne: the number of bad results (the limit. It varies together with ns)

To avoid a “final undecided” in the standard test, a second limit must be introduced and the single decision co-ordinate (ne,ns) needs a high ne, leading to a fixed (high) test time. In the early decision concept, having the same selectivity and the same confidence level an “undecided” need not to be avoided, as it can be decided later. A perfect DUT will hit the decision coordinate (ne,ns) with ne=0. This test time is short.

## G.7.7	Standard test versus early decision concept

For Supplier Risk:

The wrong decision probability D in the standard test is the probability, to decide a DUT in-correct in the single decision point. In the early decision concept there is a probability of in-correct decisions d at each point of the limit-curve. The sum of all those wrong decision probabilities accumulate to D. Hence d<D

For Customer Risk:

The correct decision probability CL in the standard test is the probability, to decide a DUT correct in the single decision point. In the early decision concept there is a probability of correct decisions cl at each point of the limit-curve. The sum of all those correct decision probabilities accumulate to CL. Hence cl<CL or d>D

## G.7.8	Selectivity

There is no statistical test which can discriminate between a limit DUT and a DUT which is an (ε0) apart from the limit in finite time and high confidence level CL. Either the test discriminates against one limit with the results pass (with CL)/undecided or fail (with CL)/undecided, or the test ends in a result pass (with CL)/fail (with CL) but this requires a second limit.

For CL>1/2, a (measurement-result = specified-DUT-quality), generates undecided in test “supplier risk against pass limit” (a, from above) and also in the test “customer risk against the fail limit “ (aa)

For CL>1/2, a DUT, known to be on the limit, will be decided pass for the test “customer risk against pass limit” (b) and also “supplier risk against fail limit” (bb).

This overlap or undecided area is not a fault or a contradiction, however it can be avoided by introducing a Bad or a Good DUT quality according to:

-	Bad DUT quality: specified DUT-quality * M (M>1)

-	Good DUT quality: specified DUT-quality * m (m<1)

Using e.g. M>1 and CL=95% the test for different DUT qualities yield different pass probabilities:

Figure G.X.8-1: Pass probability versus DUT quality

## G.7.9	Design of the test

The receiver characteristic test are defined by the following design principles:

1.	The early decision concept is applied.

2.	A second limit is introduced: Bad DUT factor M>1

3.	To decide the test pass:

Supplier risk is applied based on the Bad DUT quality

To decide the test fail

Customer Risk is applied based on the specified DUT quality

The receiver characteristic test are defined by the following parameters:

1.	Limit ER = 0.05

2.	Bad DUT factor M=1.5 (selectivity)

3.	Confidence level CL = 95% (for specified DUT and Bad DUT-quality)

This has the following consequences:

1.	A measurement on the fail limit is connected with 2 equivalent statements:

| A measurement on the fail-limit shows, that the DUT is worse than the specified DUT-quality | A DUT, known have the specified quality, shall be measured and decided pass |
| --- | --- |

2.	A measurement on the pass limit is connected with the complementary statements:

| A measurement on the pass limit shows, that the DUT is better than the Bad DUT-quality. | A DUT, known to have the Bad DUT quality, shall be measured and decided fail |
| --- | --- |

The left column is used to decide the measurement.

The right column is used to verify the design of the test by simulation.

The simulation is based on the two fulcrums A and B only in Figure G.x.8-1

3.	Test time

The minimum and maximum test time is fixed.

The average test time is a function of the DUT’s quality.

The individual test time is not predictable.

4.	The number of decision co-ordinates (ne,ns) in the early decision concept is responsible for the selectivity of the test and the maximum test time. Having fixed the number of decision co-ordinates there is still freedom to select the individual decision co-ordinates in many combinations, all leading to the same confidence level.

## G.7.10	Simulation to derive the pass fail limits in Table G.2.4-1

There is freedom to design the decision co-ordinates (ne,ns).

The binomial distribution and its inverse is used to design the pass and fail limits. Note that this method is not unique and that other methods exist.

Where

-	fail(..) is the error ratio for the fail limit

-	pass(..) is the error ratio for the pass limit

-	ER is the specified error ratio 0.05

-	ne is the number of bad results. This is the variable in both equations

-	M is the Bad DUT factor M=1.5

-	df is the wrong decision probability of a single (ne,ns) co-ordinate for the fail limit. 
	It is found by simulation to be df = 0.006

-	clp is the confidence level of a single (ne,ns) co-ordinate for the pass limit.
It is found by simulation to be clp = 0.9945

-	qnbinom(..): The inverse cumulative function of the negative binomial distribution

The simulation works as follows:

-	A large population of limit DUTs with true ER = 0.05 is decided against the pass and fail limits.

-	clp    and df   are tuned such that CL (95%) of the population passes and D (5%) of the population fails.

-	A population of Bad DUTs with true ER = M*0.05 is decided against the same pass and fail limits.

-	clp    and df   are tuned such that CL (95%) of the population fails and D (5%) of the population passes.

-	This procedure and the relationship to the measurement is justified in clause G.x.9. The number of DUTs decrease during the simulation, as the decided DUTs leave the population. That number decreases with an approximately exponential characteristics. After 146 bad results all DUTs of the population are decided.

NOTE:	The exponential decrease of the population is an optimal design goal for the decision co-ordinates (ne,ns), which can be achieved with other formulas or methods as well.

# G.8	Statistical Testing of V2V and V2X Performance Requirements – Non concurrent with E-UTRA

## G.8.1	General

The system simulator (SS) sends sidelink packets to the UE under test. The number of packets sent by the SS is predefined by the test time in G.8.3 and G.8.4 for requirements with throughput and probability of miss-detection, respectively.

The UE is required to keep an updated counter of the received PSCCH and PSSCH transport blocks and to report either PSCCH, PSSCH and/or STCH PDCP SDU metrics to the SS upon demand via AT command +CUSPCREQ [7]. The SS can use the reported metrics to determine a Pass or Fail decision.

Three state variables have been defined for this purpose:

1)	PSCCH_PACKET_COUNTER: to count the number of received PSCCH transport blocks

2)	PSSCH_PACKET_COUNTER: to count the number of received PSSCH transport blocks

3)	STCH_PACKET_COUNTER: to count the number of received STCH PDCP SDUs

The UE shall increase the respective counters upon successful reception of the corresponding transport block. To manage these counters, it is required to close the UE test loop back mode E. Such operation is performed following an AT-command based procedure to set the UE in State 5A-V2X [7]. The counters are initialized to zero upon closure of the UE test loop back mode E.

## G.8.2	Test Method for Performance Test Cases

Figure G.8.2-1 descriptively represents the course of a test based in two metrics: the noise-normalized test SNR and the performance session, represented in terms of fractional throughput. The SS shall read the sidelink packet counters twice and calculate the performance metrics based on the sample difference between both read operations. This effectively results on the discard of early sidelink counter values that might take place between their initialization at State 5A-V2X and the stabilized performance session. The test method is as follows:

1)	The UE is set on State 5A-V2X, according to the procedure defined in [7]. In this process both PSCCH, PSSCH and STCH PDCP SDU counters are initialized to zero. This takes place during the initial conditions stage.

2)	Once the UE is operating on State 5A-V2X, the SNR is set to the test SNR level and the scheduling of sidelink packets (PSCCH, PSSCH and/or STCH PDCP SDU) starts. This takes place during the test procedure stage.

3)	Once the scheduling of sidelink packets starts, the SS shall wait for a stability window of T=10 seconds before the first PSCCH/PSSCH/STCH PDCP SDU sample counter read.

4)	Upon expiration of the stability window, the SS shall perform the first PSCCH/PSSCH/STCH PDCP SDU sample counter read (Time Point A in Figure G.8.2-1) and record the retrieved values. Then the performance session starts.

5)	During the performance session the SS shall record the number of transmitted sidelink packets (PSCCH, PSSCH and/or STCH PDCP SDU) starting in Time Point A and ending in Time Point B. The performance session lasts for the Test Time defined in sections G.8.3 and G.8.4 for throughput and probability of miss-detection, respectively.

6)	Once the Test Time is completed the SS shall perform a second PSCCH/PSSCH/STCH PDCP SDU sample counter read (Time Point B in Figure G.8.2-1) and record the retrieved values. The SS shall use the retrieved counter values at Time Point A and Time Point B, as well as the recorded number of transmitted sidelink packets (PSCCH, PSSCH and/or STCH PDCP SDU) between Time Point A and Time Point B, to calculate the PSCCH probability of miss-detection, the PSSCH BLER and the STCH PDCP SDU loss, respectively.

Figure G.8.2-1: Proposed Test Method for V2V/V2X Performance Test Cases

According to both the test method described in this section and Figure G.8.2-1 the PSCCH probability of miss-detection can be defined as follows:

Similarly, we can define the PSSCH BLER as follows:

Finally, we can define the STCH PDCP SDU loss as follows:

Where:

PMDPMD = Probability of Miss-Detection

BLER = Block Error Rate

LossPSCCHCounterSTCH_SDU = STCH PDCP SDU Loss rate

PSCCHCounterPSCCHCounter = PSCCH_PACKET_COUNTER

PSSCHCounterPSSCHCounter = PSSCH_PACKET_COUNTER

PSCCHCounterSTCH_SDUCounter = STCH_PACKET_COUNTER

T = 10 seconds stability window

PSCCHTransmitted = Transmitted number of PSCCH transport blocks counted by the SS

PSSCHTransmitted = Transmitted number of PSSCH transport blocks counted by the SS

STCH_SDUTransmitted = Transmitted number of STCH PDCP SDUs counted by the SS

MNAS = Minimum Number of Active Subframes, as per Table G.8.3-1 and Table G.8.4-1 for performance tests with throughput and probability of miss-detection, respectively

## G.8.3	Test Time for V2V and V2X Performance Requirements with Throughput

The throughput requirements are tested in terms of PSSCH BLER. The maximum BLER requirement for all V2V and V2X tests is 10%. The tests are performed in a variety of AWGN and fading propagation models. For each test in Table G.8.3-1 the Test Time in Subframes is to be applied to the test method described in section G.8.2.

Table G.8.3-1: Minimum Test Time for V2V and V2X Performance Requirements with Throughput

| Test No | Performance Scenario | Minimum Number of Active Subframes (MNAS) | Minimum Number of Subframes (MNS)(Note 1) | Test Time in Subframes (Note 2) |
| --- | --- | --- | --- | --- |
| 14.2 Test 1 | CD.8 (20 MHz, 8 PRB, 16QAM 1/2) (1x2 Low) EVA 180 1 PSSCH Transmission | 20 000 (MNAS not simulated, but conservatively estimated based on existing EVA fading scenarios with close but smaller Doppler spread) | 20 000 | 20 000 |
| 14.2 Test 2 | CD.9 (10 MHz, 3 PRB, QPSK 1/3) (1x2 Low) EVA 2700 2 PSSCH Transmissions | 15 000 (MNAS not simulated, but conservatively estimated based on existing EVA fading scenarios with close but smaller Doppler spread) | 15 000 | 30 000 |
| 14.4 Test 1 | CD.10 (20 MHz, 3 PRB, QPSK 3/4) (1x2) AWGN 1 PSSCH Transmission | 30 000 (MNAS not simulated, but conservatively estimated based on existing scenarios) | 30 000 | 30 000 |
| Note 1: The Minimum Number of Subframes is the total minimum number of subframes (active and inactive) required for this demodulation scenario and is derived from the MNAS, according to the resource pool configuration defined for each test case, [5] section 6.3.8 and to [10] section 14.1.5. Note 2: The Test Time is based on the Minimum Number of Subframes (MNS) according to the formula: Test Time in Subframes = 1000*CEIL(MNS/1000). For tests with more than 1 PSSCH Transmission, the formula is: Test Time in Subframes = 1000*CEIL(MNS/1000)*Number of Transmissions. |  |  |  |  |

## G.8.4	Test Time for V2V and V2X Performance Requirements with Probability of Miss-Detection

The probability of miss-detection requirements are tested in terms of Probability of PSCCH miss-detection. The maximum Probability of PSCCH miss-detection for all V2V and V2X tests is 1%. The tests are performed in a variety of Static, AWGN and fading propagation models. For each test in Table G.8.4-1 the Test Time in Subframes is to be applied to the test method described in section G.8.2.

Table G.8.4-1: Minimum Test Time for V2V and V2X Performance Requirements with Probability of Miss-Detection

| Test No | Performance Scenario | Minimum Number of Active Subframes (MNAS) | Minimum Number of Subframes (MNS)(Note 1) | Test Time in Subframes (Note 2) |
| --- | --- | --- | --- | --- |
| 14.3 Test 1 | CC.8 (10 MHz, 2 PRB, QPSK 1/3) (1x2 Low) EVA 1500 1 PSCCH Transmission | 15 000 (MNAS not simulated, but conservatively estimated based on existing fading scenarios with close but smaller Doppler spread) | 15 000 | 15 000 |
| 14.8 Test 1 Test 2 | CC.8 (20 MHz, 2 PRB, QPSK 1/3) (1x2) Static Channel 1 PSCCH Transmission | 30 000 (MNAS not simulated, but conservatively estimated based on existing fading scenarios) | 30 000 | 30 000 |
| Note 1: The Minimum Number of Subframes is the total minimum number of subframes (active and inactive) required for this demodulation scenario and is derived from the MNAS, according to the resource pool configuration defined for each test case, [5] section 6.3.8 and to [10] section 14.1.5. Note 2: The Test Time is based on the Minimum Number of Subframes (MNS) according to the formula: Test Time in Subframes = 1000*CEIL(MNS/1000). |  |  |  |  |

Annex H (normative):
Uplink Physical Channels

# H.0	Uplink Signal Levels

Uplink signal power is a UE figure, which is configured by the Test System by means of:

RRC messages (IE-s), such as:

-	UplinkPowerControlCommon (-v1020, SCell-r10),

-	UplinkPowerControlDedicated (-v1020, SCell-r10),

-	Other IE-s affecting directly or indirectly the uplink power,

and L1/2 Power control commands (TPC).

The uplink power settings are specified in the test case.

Otherwise, the uplink power settings result from the default RRC messages described in 3GPP TS 36.508 [7], and appropriate TPC-s, which are sent to the UE to transmit with an UL power level necessary for maintaining the call during the test.

## H.0.1	Uplink Signal Levels for NB1

Uplink signal power is a UE figure, which is configured by the Test System by means of:

RRC messages (IE-s), such as:

-	NPUSCH-ConfigCommon-NB

-	NPUSCH-ConfigDedicated-NB

-	UplinkPowerControl-NB

-	Other IE-s affecting directly or indirectly the uplink power,

The uplink power settings are specified in the test case.

Otherwise, the uplink power settings result from the default RRC messages described in 3GPP TS 36.508 [7], which are sent to the UE to transmit with an UL power level necessary for maintaining the call during the test.

# H.1	General

This annex specifies the uplink physical channels that are needed for setting a connection and channels that are needed during a connection. Table H.1-1 describes the mapping of uplink physical channels and signals to physical resources for FDD. Table H.1-2 describes the mapping of uplink physical channels and signals to physical resources for TDD.

Table H.1-1: Mapping of uplink physical channels and signals to physical resources for FDD

| Physical channel | Time Domain Location | Frequency Domain Location | Note |
| --- | --- | --- | --- |
| PRACH | Allowed for the parameter prach-Configuration Index provided by higher layers | Allowed for the parameter prach-FrequencyOffset provided by higher layers | Mapping rule is specified in TS 36.211 [8] Section 5.7.1 |
| DMRS | For PUCCH: Symbols 2 to 4 of each slot (PUCCH format: 1, 1a, 1b) Symbol 1 and 5 of each slot (PUCCH format: 2, 2a, 2b) For PUSCH: Symbol 3 of each slot | Uplink system bandwidth dependent. | Mapping rule of DMRS for PUCCH is specified in TS 36.211 [8] 5.5.2.2.2 Mapping rule of DMRS for PUSCH is specified in TS 36.211 [8] 5.5.2.1.2 |
| PUCCH | Slot 0 and 1 of each subframe | Each 12 subcarriers of both ends of the bandwidth | Mapping rule is specified in TS 36.211 [8] Section 5.4.3 |
| PUSCH | All remaining SC-FDMA symbols of each subframe not allocated to DMRS | RBs allocated according to Reference Measurement channel in Annex A.2 | Mapping rule is specified in TS 36.211 [8] Section 5.4.2 |
| SRS | Allowed for the cell-specific parameter srs-BandwidthConfig and the UE-specific parameter srs-Bandwidth provided by higher layers | Allowed for the cell-specific parameter srsMaxUpPt and the UE-specific parameter transmissionComb or transmissionComb-ap provided by higher layers | Mapping rule is specified in TS 36.211 [8] Section 5.5.3.2 |

Table H.1-2: Mapping of uplink physical channels and signals to physical resources for TDD

| Physical channel | Time Domain Location | Frequency Domain Location | Note |
| --- | --- | --- | --- |
| PRACH | Allowed for the parameters in prach-Configuration Index provided by higher layers | For format 0-3, the frequency location allowed is by prach-FrequencyOffset and in prach-Configuration Index provided by higher layers. Preamble format 4 is mapped only on UpPTS, where the frequency location allowed is only by in prach-Configuration Index provided by higher layers. | Mapping rule is specified in TS 36.211 [8] Section 5.7.1 |
| DMRS | For PUCCH: Symbols 2 to 4 of each slot (PUCCH format: 1, 1a, 1b) Symbol 1 and 5 of each slot (PUCCH format: 2, 2a, 2b) For PUSCH: Symbol 3 of each slot | Uplink system bandwidth dependent. | Mapping rule of DMRS for PUCCH is specified in TS 36.211 [8] 5.5.2.2.2 Mapping rule of DMRS for PUSCH is specified in TS 36.211 [8] 5.5.2.1.2 |
| PUCCH | Slot 0 and 1 of each subframe | Each 12 subcarriers of both ends of the bandwidth | Mapping rule is specified in TS 36.211 [8] Section 5.4.3 |
| PUSCH | All remaining SC-FDMA symbols of each subframe not allocated to DMRS | RBs allocated according to Reference Measurement channel in Annex A.2 | Mapping rule is specified in TS 36.211 [8] Section 5.4.2 |
| SRS | Allowed for the cell-specific parameter srs-BandwidthConfig and the UE-specific parameter srs-Bandwidth provided by higher layers | Allowed for the cell-specific parameter srsMaxUpPt and the UE-specific parameter transmissionComb or transmissionComb-ap provided by higher layers | Mapping rule is specified in TS 36.211 [8] Section 5.5.3.2 |

NOTE:	PUSCH, PUCCH, DMRS are not present in UpPTS for TDD.

## H.1.1	General for NB1

This annex specifies the uplink physical channels that are needed for setting a connection and channels that are needed during a connection. Table H.1.1-1 describes the mapping of uplink physical channels and signals to physical resources for NB1 UE.

Table H.1.1-1: Mapping of uplink physical channels and signals to physical resources for NB1

| Physical channel | Time Domain Location | Frequency Domain Location | Note |
| --- | --- | --- | --- |
| NPRACH | Allowed for the parameter nprach-Configuration Index provided by higher layers | Allowed for the parameter nprach-SubcarrierOffset provided by higher layers | Mapping rule is specified in TS 36.211 [8] Section 10.1.6 |
| NPUSCH | All the SC-FDMA symbols of each subframe. | RUs allocated according to Reference Measurement channel in Annex A.2 | Mapping rule is specified in TS 36.211 [8] Section 10.1.3 |

# H.2	Set-up

Table H.2-1 describes the uplink physical channels that are required for connection set up.

Table H.2-1: Uplink Physical Channels required for connection set-up

Physical Channel

PRACH

DMRS

PUCCH

PUSCH

## H.2.1	Set-up for NB1

Table H.2.1-1 describes the uplink physical channels that are required for connection set up.

Table H.2.1-1: Uplink Physical Channels required for connection set-up

Physical Channel

NPRACH

NPUSCH

# H.3	Connection

The following clauses describes the uplink physical channels that are transmitted during a connection i.e., when measurements are done.

## H.3.0	Measurement of Transmitter Characteristics

As specified in the test case. Otherwise:

-	PUSCH + DMRS for PUSCH (and DMRS) measurements.

-	PUCCH + DMRS for PUCCH (and DMRS) measurements.

-	PRACH for PRACH measurements.

SRS for SRS measurements.

## H.3.1	Measurement of Receiver Characteristics

As specified in the test case. Otherwise:

- PUSCH + DMRS for measurements with uplink interference configured.

- PUCCH + DMRS for measurements without uplink interference configured.

## H.3.2	Measurement of Performance Requirements

As specified in the test case. Otherwise:

- PUCCH + DMRS for measurements without CSI feedback, or with CSI feedback in PUCCH mode.

- PUSCH + DMRS for measurements with CSI feedback in PUSCH mode.

# H.4	Connection for NB1

The following clauses describes the uplink physical channels that are transmitted during a connection i.e., when measurements are done.

## H.4.0	Measurement of Transmitter Characteristics

As specified in the test case. Otherwise:

-	NPUSCH for measurements.

-	NPRACH for PRACH measurements.

## H.4.1	Measurement of Receiver Characteristics

As specified in the test case. Otherwise:

- NPUSCH (format 2) for measurements.

## H.4.2	Measurement of Performance Requirements

As specified in the test case. Otherwise:

- NPUSCH (format 2) for measurements.

Annex I (informative):
Handling requirements and tests for different releases and UE capabilities

This annex gives guidance on how minimum requirements in different releases of 3GPP TS 36.101 [2] and different UE capabilities are handled in the specification 3GPP TS 36.521-1.

# I.1	General Considerations

Conformance tests in TS 36.521-1 are derived and specified by RAN WG5 based on minimum requirements in TS 36.101 [2] defined by RAN WG4. The actual practice of RAN WG5 is to specify conformance tests only in a single release of TS 36.521-1, capturing requirements defined by RAN WG4 in all releases of TS 36.101 [2].

1)	In general the structure of TS 36.521-1 follows the structure of TS 36.101 [2].

2)	In general for minimum requirements specified in a single clause in TS 36.101 [2], a corresponding conformance test is specified in a single clause in TS 36.521-1.

3)	Exceptions to rule 2) occur in the following cases:

a)	Coverage of minimum requirements is too wide, in term of different test environments required for verifying all of them.

b)	Coverage of minimum requirements is too wide, in term of different features which need to be tested for verifying all of them.

c)	Coverage of minimum requirements is too wide, in term of different UE capabilities required for verifying all of them.

A set of conformance tests is generated in TS 36.521-1 based on the same clause with minimum requirements in TS 36.101 [2]. This "test group" consists in several "individual tests".

4)	Exceptions to rule 2) occur also in the case when minimum requirements (including test conditions, test points etc.) are not the same in different releases of TS 36.101 [2]. More in details the differences may consist in:

a)	Different requirement values / test conditions for the same test points.

b)	Exclusion / Replacement / Addition of test points without introduction of new features

c)	Addition of test points introducing new features.

A set of conformance tests is generated in TS 36.521-1 based on the same clause with minimum requirements in TS 36.101 [2]. This set consists in one "original test" and one or several "additional tests".

5)	The clause-number and the title of the "test group" / "original test" is derived based on the number and title of the corresponding minimum requirements in TS 36.101 [2], while the clause-numbers and titles of the "individual tests" / "additional tests" are derived based on those of the "test group" / "original test". In any case it should be avoided to have in TS 36.521-1 specification tests with the same title, even though they have different clause-numbers.

6)	In case of high similarity between "individual tests", or between "additional tests" and "original test", in order to reduce the standardization and maintenance work the content of clauses for "individual tests" / "additional tests" may be reduced to a minimum by referencing to the analogue clauses of other "individual tests" / the "original test" and specifying the exceptions (requirement-tables, test parameter tables etc). This method should be applied very carefully considering differences in core functionalities of different E-UTRAN releases.

Details how to apply the above principles to concrete scenarios are defined in Annex I.2.

# I.2	Concrete scenarios

## I.2.1	Tests for minimum requirements varying between releases, without introduction of new features

Different minimum requirement between different releases of 3GPP TS 36.101 [2] without introduction of new features represent scenarios according to Annex I.1 bullets 4) a) and b).

In TS 36.521-1 are specified one "original test" and several "additional tests" respectively applicable to the appropriate releases. This is shown graphically in Figure I.2.1-1.

Figure I.2.1-1: Mapping of minimum requirements to conformance tests, when minimum requirements are specified in multiple releases

Rule:

Following tests and clauses are specified in TS 36.521-1:

Original tests:

<x.x.x>	<Test>

Additional tests:

<x.x.x>_<y>	<Test> (<Release> <UE capability>)

where:

<x.x.x>	= number of the original test

<y>	= incrementing clause number (Arabic numeral)

<Test>	= title of the original test

<Release>	= optional identifier showing the release applicability of the test. To be included only when there is already a test with the same title for earlier releases.

<UE capability>	= optional identifier showing the UE capability, which leads to additional test. To be included only when there is already a test with the same title for the same release.

Examples:

1.2.3		Virtual test

1.2.3_1	Virtual test (Rel-9 and forward)

1.2.3_2	Virtual test (Rel-10 and forward)

## I.2.2	Tests for CA (Carrier aggregation)

### I.2.2.1	CA Tx tests (Chapter 6)

CA Transmitter minimum requirements in 3GPP TS 36.101 [2] are already specified in separate clauses from the legacy requirements, bearing the suffix "A" in the number and "for CA" in the title. However minimum requirements have a wide coverage in terms of Annex I.1 bullet 3).

In TS 36.521-1 are specified several separate "individual tests" for different CA scenarios and UE capabilities.

Rule:

Following tests and clauses are specified in TS 36.521-1:

Test group clause:

<x.x.x>A	<Test> for CA

Individual tests:

<x.x.x>A.<y>	<Test> for CA (<CA type> <DL/UL support> <BW Class> <Release>)

where:

<x.x.x>	= number of the corresponding legacy test (if available), or number derived from TS 36.101 without potential suffix "A" (if no legacy test available)

<y>	= incrementing clause number (Arabic numeral)

<Test>	= title of the corresponding legacy test (if available), or title derived from TS 36.101 without potential suffix "for CA" (if no legacy test available)

<CA type>	= mandatory identifier {intra-band contiguous; intra-band non-contiguous; inter-band}

<DL/UL support>= mandatory identifier {DL CA without UL CA; DL CA and UL CA}

<BW Class>	= optional identifier showing UE CA Bandwidth class {Class B; Class C; Class D; Class E; Class F}. To be included only for intra-band contiguous scenarios, if there is already a test with the same title.

<Release>	= optional identifier showing the release applicability of the test. To be included only when there is already a test with the same title for earlier releases.

Examples:

1.2.3		Virtual test

1.2.3A	Virtual test for CA

1.2.3A.1	Virtual test for CA (intra-band contiguous DL CA and UL CA)

1.2.3A.2	Virtual test for CA (inter-band DL CA and UL CA)

1.2.3A.3	Virtual test for CA (intra-band non-contiguous DL CA and UL CA)

1.2.3A.4	Virtual test for CA (3UL CA)

### I.2.2.2	CA Rx tests (Chapter 7)

Same as for Transmitter tests (Chapter 6) in Annex I.2.2.1 with following exception:

Examples:

1.2.3	Virtual test

1.2.3A	Virtual test for CA

1.2.3A.1	Virtual test for CA (intra-band contiguous DL CA and UL CA)

1.2.3A.2	Virtual test for CA (intra-band contiguous DL CA without UL CA)

1.2.3A.3	Virtual test for CA (inter-band DL CA without UL CA)

1.2.3A.4	Virtual test for CA (intra band non-contiguous DL CA without UL CA)

1.2.3A.5	Virtual test for CA (3DL CA)

1.2.3A.6	Virtual test for CA (inter-band DL CA and UL CA)

1.2.3A.7	Virtual test for CA (4DL CA)

1.2.3A.8	Virtual test for CA (5DL CA)

### I.2.2.3	CA Performance tests (Chapter 8)

CA Performance minimum requirements in 3GPP TS 36.101 [2] are specified mostly in the same clause with the legacy requirements, as additional test points marked with CA capability identifiers. CA introduces a new feature in terms of Annex I.1 bullet 4) c). Furthermore the set of CA test points has a wide coverage in terms of Annex I.1 bullet 3).

In TS 36.521-1 are specified several separate "individual tests" for different CA scenarios and UE capabilities.

Rule:

Following tests and clauses are specified in TS 36.521-1:

Test group clause:

<x.x.x>_A	<Test> for CA

Individual tests:

<x.x.x>_A.<y>	<Test> for CA (<CA type > <DL/UL support> <Release>)

where:

<x.x.x>	= number of the corresponding legacy test (if available), or number derived from TS 36.101 without potential suffix "A" (if no legacy test available)

<y>	= incrementing clause number (Arabic numeral)

<Test>	= title of the corresponding legacy test (if available), or title derived from TS 36.101 without potential "for CA" (if no legacy test available)

<CA type>	= mandatory identifier {intra-band contiguous; intra-band non-contiguous; inter-band}

<DL/UL support>	= mandatory identifier {DL CA}

<Release>	= optional identifier showing the release applicability of the test. To be included only when there is already a test with the same title for earlier releases.

Examples:

1.2.3		Virtual test

1.2.3_A		Virtual test for CA

1.2.3_A.1	Virtual test for CA (2DL CA)

1.2.3_A.2	Virtual test for CA (3DL CA)

1.2.3_A.3	Virtual test for CA (4DL CA)

1.2.3_A.4	Virtual test for CA (5DL CA)

## I.2.3	Tests for UL-MIMO (Uplink Multiple Antenna Transmission)

### I.2.3.1	UL-MIMO Tx tests (Chapter 6)

UL-MIMO Transmitter minimum requirements in 3GPP TS 36.101 [2] are already specified in separate clauses from the legacy requirements, bearing the suffix "B" in the number and "for UL-MIMO" in the title.

Rule:

Following tests and clauses are specified in TS 36.521-1:

Test group clause:

<x.x.x>B	<Test> for UL-MIMO

Individual tests:

<x.x.x>B.<y>	<Test> for UL-MIMO (<Release>)

where:

<x.x.x>	= number of the corresponding legacy test (if available), or number derived from TS 36.101 without potential suffix "B" (if no legacy test available)

<y>	= incrementing clause number (Arabic numeral)

<Test>	= title of the corresponding legacy test (if available), or title derived from TS 36.101 without potential suffix "for UL-MIMO" (if no legacy test available)

<Release>	= optional identifier showing the release applicability of the test. To be included only when there is already a test with the same title for earlier releases.

Examples:

1.2.3		Virtual test

1.2.3B	Virtual test for UL-MIMO

### I.2.3.2	UL-MIMO Rx tests (Chapter 7)

Same as for Transmitter tests (Chapter 6) in Annex I.2.3.1.

## I.2.4	Tests for eDL-MIMO (Enhanced Downlink Multiple Antenna Transmission)

### I.2.4.1	eDL MIMO Performance tests (Chapter 8)

eDL MIMO Performance minimum requirements in 3GPP TS 36.101 [2] are specified in separate clauses from the legacy requirements. In some cases the minimum requirements have a wide coverage in terms of Annex I.1 bullet 3) a) with respect of the presence of a simultaneous interfering transmission a) with respect of the presence of a simultaneous interfering transmission.

In TS 36.521-1 are specified several separate "individual tests" for eDL-MIMO.

Rule:

Following tests and clauses are specified in TS 36.521-1:

Test group clause:

<x.x.x>_D	<Test> for eDL-MIMO

Individual tests:

<x.x.x>_D.<y>	<Test> for eDL-MIMO (<Release>)

where:

<x.x.x>	= number of the corresponding legacy test (if available), or number derived from TS 36.101 without potential suffix (if no legacy test available).

<y>		=optional incrementing clause number (Arabic numeral) (if many individual tests).

<Test>	= title of the corresponding legacy test (if available), or title derived from TS 36.101 without potential suffix potential suffix (if no legacy test available).

<Release>		= optional identifier showing the release applicability of the test. To be included only when there is already a test with the same title for earlier releases.

Examples:

1.2.4		Virtual test

1.2.4_D	Virtual test for eDL-MIMO

### I.2.4.2	eDL MIMO CSI tests (Chapter 9)

eDL MIMO CSI minimum requirements in 3GPP TS 36.101 [2] are specified in separate clauses from legacy requirements.

Rule:

Following tests and clauses are specified in TS 36.521-1:

Test group clause:

<x.x.x>_D	<Test> for eDL-MIMO

Individual tests:

<x.x.x>_D.<y>	<Test> for eDL-MIMO (<Release>)

where:

<x.x.x>	= number of the corresponding legacy test (if available), or number derived from TS 36.101 without potential suffix (if no legacy test available).

<y>	=optional incrementing clause number (Arabic numeral) (if many individual tests).

<Test>	= title of the corresponding legacy test (if available), or title derived from TS 36.101 without potential suffix (if no legacy test available).

<Release>	= optional identifier showing the release applicability of the test. To be included only when there is already a test with the same title for earlier releases.

Examples:

1.2.4	Virtual test

1.2.4_D	Virtual test for eDL-MIMO

## I.2.5	Tests for eICIC (Enhanced Intercarrier Interference Cancellation / Coordination)

### I.2.5.1	eICIC Performance tests (Chapter 8)

eICIC Performance minimum requirements in 3GPP TS 36.101 [2] are specified in separate clauses from the legacy requirements. The set of eICIC test points have a wide coverage in terms of Annex I.1 bullet 3)a) with respect of the ABS type (Non-MBSFN / MBSFN). In TS 36.521-1 are specified several separate "individual tests" for eICIC.

Rule:

Following tests and clauses are specified in TS 36.521-1:

Test group clause:

<x.x.x>_C	<Test> for eICIC

Individual tests:

<x.x.x>_C.<y>	<Test> for eICIC (<ABS-type> <Release>)

where:

<x.x.x>	= number of the corresponding legacy test (if available), or number derived from TS 36.101 without potential suffix (if no legacy test available).

<y>		=optional incrementing clause number (Arabic numeral) (if many individual tests).

<Test>	= title of the corresponding legacy test (if available), or title derived from TS 36.101 without potential suffix (if no legacy test available).

<ABS-Type>	= mandatory identifier {non-MBSFN ABS, MBSFN ABS}

<Release>		= optional identifier showing the release applicability of the test. To be included only when there is already a test with the same title for earlier releases.

Examples:

1.2.4		Virtual test

1.2.4_C	Virtual test for eICIC

1.2.4_C.1	Virtual test for eICIC (non-MBSFN ABS)

1.2.4_C.2	Virtual test for eICIC (MBSFN ABS)

### I.2.5.2	eICIC CSI tests (Chapter 9)

Same as for Performance tests (Chapter 8) in Annex I.2.5.1.

## I.2.6	Tests for feICIC (Further Enhanced Non CA-based ICIC for LTE)

### I.2.6.1	feICIC Performance tests (Chapter 8)

feICIC Performance minimum requirements in 3GPP TS 36.101 [2] are specified in separate clauses from the legacy requirements. The set of feICIC test points have a wide coverage in terms of Annex I.1 bullet 3)a) with respect of the ABS type (Non-MBSFN / MBSFN). In TS 36.521-1 are specified several separate "individual tests" for feICIC.

Rule:

Following tests and clauses are specified in TS 36.521-1:

Test group clause:

<x.x.x>_E	<Test> for feICIC

Individual tests:

<x.x.x>_E.<y>	<Test> for feICIC (<ABS-type> <Release>)

where:

<x.x.x>	= number of the corresponding legacy test (if available), or number derived from TS 36.101 without potential suffix (if no legacy test available).

<y>		=optional incrementing clause number (Arabic numeral) (if many individual tests).

<Test>	= title of the corresponding legacy test (if available), or title derived from TS 36.101 without potential suffix (if no legacy test available).

<ABS-Type>	= mandatory identifier {non-MBSFN ABS, MBSFN ABS}

<Release>		= optional identifier showing the release applicability of the test. To be included only when there is already a test with the same title for earlier releases.

Examples:

1.2.6		Virtual test

1.2.6_E	Virtual test for feICIC

1.2.6_E.1	Virtual test for feICIC (non-MBSFN ABS)

1.2.6_E.2	Virtual test for eICIC (MBSFN ABS)

### I.2.6.2	feICIC CSI tests (Chapter 9)

Same as for Performance tests (Chapter 8) in Annex I.2.6.1.

## I.2.7	Tests for DL ComP (Downlink Coordinated Multipoint Transmission)

### I.2.7.1	CoMP Performance tests (Chapter 8)

CoMP Performance minimum requirements in 3GPP TS 36.101 [2] are specified in separate clauses from the legacy requirements. The set of CoMP test points may have a wide coverage in terms of Annex I.1 bullet 3)a) with respect of the number of CSI processes (Single / Multiple CSI process) etc. In such cases in TS 36.521-1 are specified several separate "individual tests" for CoMP.

Rule:

Following tests and clauses are specified in TS 36.521-1:

Test (group) clause:

<x.x.x>_F		<Test> for CoMP

Individual tests:

<x.x.x>_F.<y>	<Test> < CSI Process number > for CoMP (<Release>)

where:

<x.x.x>						= number derived from TS 36.101.

<y>							= optional incrementing clause number (Arabic numeral) (if many individual tests).

<Test>						= title derived from TS 36.101.

<CSI Process number>	= optional identifier {with Single CSI process, with Multiple CSI processes} (if many individual tests)

<Release>	= optional identifier showing the release applicability of the test. To be included only when there is already a test with the same title for earlier releases.

Examples:

1.2.3	Virtual test

1.2.3_F	Virtual test for CoMP

1.2.3_F.1	Virtual test with Single CSI process for CoMP

1.2.3_F.2	Virtual test with Multiple CSI processes for CoMP

### I.2.7.2	CoMP CSI tests (Chapter 9)

Same as for Performance tests (Chapter 8) in Annex I.2.7.1.

## I.2.8	Tests for Further eDL-MIMO (Enhanced Downlink Multiple Antenna Transmission) Enhancement for LTE-A

### I.2.8.1	eDL MIMO Enhancement CSI tests (Chapter 9)

Further eDL MIMO Enhancement for LTE-A CSI minimum requirements in 3GPP TS 36.101 [2] are specified in separate clauses from the legacy requirements.

Rule:

Following tests and clauses are specified in TS 36.521-1:

<x.x.x>	<Test> for eDL-MIMO Enhancement

Individual tests:

<x.x.x>< _y>	<Test> for eDL-MIMO Enhancement (<Release>)

where:

<x.x.x>	= number derived from the corresponding requirement clause in TS 36.101.

<_y>	= optional incrementing clause number (Arabic numeral) with preceding underscore. To be applied only when there is already a test with the same title for earlier releases.

<Test>	= title derived from the corresponding requirement clause in TS 36.101.

<Release>	= optional identifier showing the release applicability of the test. To be included only when there is already a test with the same title for earlier releases.

Examples:

1.2.3	Virtual test for eDL-MIMO Enhancement

1.2.3_1	Virtual test for eDL-MIMO Enhancement (Rel-13 and forward)

## I.2.9	Tests for UE category 0

### I.2.9.1	UE category 0 Tx tests (Chapter 6)

UE category 0 specific Transmitter minimum requirements in 3GPP TS 36.101 [2] are specified in separate clauses from the legacy requirements, bearing the suffix "E" in the number and "for UE category 0" in the title. In other cases where no category 0 specific requirements exist, the legacy requirements apply. To maintain consistency all test cases for UE category 0 to be specified in TS 36.521-1 are handled similarly.

Rule:

Following tests and clauses are specified in TS 36.521-1:

<x.x.x>E	<Test> for UE category 0

<x.x.x>E<_y>	<Test> for UE category 0 (<Release>)

where:

<x.x.x>	= number of the corresponding legacy test (if available), or number derived from TS 36.101 without potential suffix "E" (if no legacy test available)

<_y>	= optional incrementing clause number (Arabic numeral) with preceding underscore. To be applied only when there is already a test with the same title for earlier releases.

<Test>	= title of the corresponding legacy test (if available), or title derived from TS 36.101 without potential "for UE category 0" (if no legacy test available)

<Release>	= optional identifier showing the release applicability of the test. To be included only when there is already a test with the same title for earlier releases.

Examples:

1.2.3		Virtual test

1.2.3E		Virtual test for UE category 0

1.2.3E_1	Virtual test for UE category 0 (Rel-13 and forward)

### I.2.9.2	UE category 0 Rx tests (Chapter 7)

Same as for Transmitter tests (Chapter 6) in Annex I.2.9.1.

### I.2.9.3	UE category 0 Performance tests (Chapter 8)

UE category 0 Performance minimum requirements in 3GPP TS 36.101 [2] are already specified in separate clauses from the legacy requirements.

Rule:

Following tests and clauses are specified in TS 36.521-1:

<x.x.x>	<Test> for UE category 0

<x.x.x><_y>	<Test> for UE category 0 (<Release>)

where:

<x.x.x>	= number derived from the corresponding requirement clause in TS 36.101

<_y>	= optional incrementing clause number (Arabic numeral) with preceding underscore. To be applied only when there is already a test with the same title for earlier releases.

<Test>	= title derived from the corresponding requirement clause in TS 36.101

<Release>	= optional identifier showing the release applicability of the test. To be included only when there is already a test with the same title for earlier releases.

Examples:

1.2.3		Virtual test for UE category 0

1.2.3_1	Virtual test for UE category 0 (Rel-13 and forward)

### I.2.9.4	UE category 0 CSI tests (Chapter 9)

Same as for Performance tests (Chapter 8) in Annex I.2.9.3.

## I.2.9A	Tests for UE category M1

### I.2.9A.1	UE category M1 Tx tests (Chapter 6)

UE category M1 specific Transmitter minimum requirements in 3GPP TS 36.101 [2] are specified in separate clauses from the legacy requirements, bearing the suffix "E" in the number and "for UE category M1" in the title. Suffix “E” is also used for UE category 0. In other cases, where no category M1 specific requirements exist, the legacy requirements apply. To maintain consistency all test cases for UE category M1 to be specified in TS 36.521-1 are handled similarly, but suffix "EA" is used to distinguish from UE category 0 test cases using suffix "E".

Rule:

Following tests and clauses are specified in TS 36.521-1:

<x.x.x>EA	<Test> for UE category M1

<x.x.x>EA<_y>	<Test> for UE category M1 (<CEMode><Release>)

where:

<x.x.x>	= number of the corresponding legacy test (if available), or number derived from TS 36.101 without potential suffix "E" (if no legacy test available)

<_y>	= optional incrementing clause number (Arabic numeral) with preceding underscore. To be applied only when there is already a test for CEModeA or a test with the same title for earlier releases.

<Test>	= title of the corresponding legacy test (if available), or title derived from TS 36.101 without potential "for UE category M1" (if no legacy test available)

<CEMode>	= optional identifier showing the CEMode applicability of the test. Can only take value CEModeB and is applied only if a corresponding CEModeA test exists.

<Release>	= optional identifier showing the release applicability of the test. To be included only when there is already a test with the same title for earlier releases.

Examples:

1.2.3		Virtual test

1.2.3EA		Virtual test for UE category M1

1.2.3EA_1	Virtual test for UE category M1 (Rel-13 and forward)

### I.2.9A.2	UE category M1 Rx tests (Chapter 7)

Same as for Transmitter tests (Chapter 6) in Annex I.2.9A.1.

### I.2.9A.3	UE category M1 Performance tests (Chapter 8)

UE category M1 Performance minimum requirements in 3GPP TS 36.101 [2] are already specified in separate clauses from the legacy requirements.

Rule:

Following tests and clauses are specified in TS 36.521-1:

<x.x.x>	<Test> for UE category M1

<x.x.x><_y>	<Test> for UE category M1 (<CEMode><Release>)

where:

<x.x.x>	= number derived from the corresponding requirement clause in TS 36.101

<_y>	= optional incrementing clause number (Arabic numeral) with preceding underscore. To be applied only when there is already a test for CEModeA or a test with the same title for earlier releases.

<Test>	= title derived from the corresponding requirement clause in TS 36.101

<CEMode>	= optional identifier showing the CEMode applicability of the test. Can only take value CEModeB and is applied only if a corresponding CEModeA test exists.

<Release>	= optional identifier showing the release applicability of the test. To be included only when there is already a test with the same title for earlier releases.

Examples:

1.2.3		Virtual test for UE category M1

1.2.3_1	Virtual test for UE category M1 (Rel-13 and forward)

### I.2.9A.4	UE category M1 CSI tests (Chapter 9)

Same as for Performance tests (Chapter 8) in Annex I.2.9A.3.

## I.2.9B	Tests for UE category M2

### I.2.9B.1	UE category M2 Tx tests (Chapter 6)

UE category M2 specific Transmitter minimum requirements in 3GPP TS 36.101 [2] are specified in separate clauses from the legacy requirements and category M1 requirements, bearing the suffix "E" in the number and "for UE category M2" in the title. Suffix “E” is also used for UE category 0. In other cases, where no category M2 specific requirements exist, the legacy requirements apply. To maintain consistency all test cases for UE category M2 to be specified in TS 36.521-1 are handled similarly, but suffix "EC" is used to distinguish from UE category 0 test cases using suffix "E", category M1 test cases using suffix "EA" and category 1bis test cases using suffix “EB”.

Rule:

Following tests and clauses are specified in TS 36.521-1:

<x.x.x>EC	<Test> for UE category M2

<x.x.x>EC<_y>	<Test> for UE category M2 (<CEMode><Release>)

where:

<x.x.x>	= number of the corresponding legacy test (if available), or number derived from TS 36.101 without potential suffix "E" (if no legacy test available)

<_y>	= optional incrementing clause number (Arabic numeral) with preceding underscore. To be applied only when there is already a test for CEModeA or a test with the same title for earlier releases.

<Test>	= title of the corresponding legacy test (if available), or title derived from TS 36.101 without potential "for UE category M2" (if no legacy test available)

<CEMode>	= optional identifier showing the CEMode applicability of the test. Can only take value CEModeB and is applied only if a corresponding CEModeA test exists.

<Release>	= optional identifier showing the release applicability of the test. To be included only when there is already a test with the same title for earlier releases.

Examples:

1.2.3		Virtual test

1.2.3EC		Virtual test for UE category M2

1.2.3EC_1	Virtual test for UE category M2 (Rel-14 and forward)

### I.2.9B.2	UE category M2 Rx tests (Chapter 7)

Same as for Transmitter tests (Chapter 6) in Annex I.2.9B.1.

### I.2.9B.3	UE category M2 Performance tests (Chapter 8)

UE category M2 Performance minimum requirements in 3GPP TS 36.101 [2] are already specified in separate clauses from the legacy requirements.

Rule:

Following tests and clauses are specified in TS 36.521-1:

<x.x.x>	<Test> for UE category M2

<x.x.x><_y>	<Test> for UE category M2 (<CEMode><Release>)

where:

<x.x.x>	= number derived from the corresponding requirement clause in TS 36.101

<_y>	= optional incrementing clause number (Arabic numeral) with preceding underscore. To be applied only when there is already a test for CEModeA or a test with the same title for earlier releases.

<Test>	= title derived from the corresponding requirement clause in TS 36.101

<CEMode>	= optional identifier showing the CEMode applicability of the test. Can only take value CEModeB and is applied only if a corresponding CEModeA test exists.

<Release>	= optional identifier showing the release applicability of the test. To be included only when there is already a test with the same title for earlier releases.

Examples:

1.2.3		Virtual test for UE category M2

1.2.3_1	Virtual test for UE category M2 (Rel-14 and forward)

### I.2.9B.4	UE category M2 CSI tests (Chapter 9)

Same as for Performance tests (Chapter 8) in Annex I.2.9B.3.

## I.2.10	Tests for SCE-L1 (Small Cell Enhancements-Physical layer)

### I.2.10.1	SCE-L1 Receiver tests (Chapter 7)

The set of 256QAM test points may have a wide coverage in terms of Annex I.1 bullet 3)c) with respect of the different UE capabilities (UE category 11-16), modulation mode and coding rate required and Annex I.1 bullet 4)a)etc with the addition of new requirements and new defined RMC for SCE-L1. In such cases in TS 36.521-1 are specified several separate "individual tests" for SCE-L1.

Rule:

Following tests and clauses are specified in TS 36.521-1:

Test (group) clause:

<x.x.x>_H	<Test> for SCE-L1

Individual tests:

<x.x.x>_H.<y>	<Test> for SCE-L1 (<Release>)

where:

<x.x.x>						= number derived from TS 36.101.

<y>							= optional incrementing clause number (Arabic numeral) (if many individual tests).

<Test>						= title derived from TS 36.101.

<Release>	= optional identifier showing the release applicability of the test. To be included only when there is already a test with the same title for earlier releases.

Examples:

1.2.3	Virtual test

1.2.3_H	Virtual test for SCE-L1

### I.2.10.2	SCE-L1 Performance tests (Chapter 8)

The set of 256QAM test points may have a wide coverage in terms of Annex I.1 bullet 3)c) with respect of the different UE capabilities (UE category 11-16) and demodulation mode required and Annex I.1 bullet 4)c)etc with the addition of new test points introducing new features of 256QAM for SCE-L1. In such cases in TS 36.521-1 are specified several separate "individual tests" for SCE-L1.

Rule:

Following tests and clauses are specified in TS 36.521-1:

Test (group) clause:

<x.x.x>_H	<Test> for SCE-L1

Individual tests:

<x.x.x>_H.<y>	<Test> for SCE-L1 (<Release>)

where:

<x.x.x>						= number derived from TS 36.101.

<y>							= optional incrementing clause number (Arabic numeral) (if many individual tests).

<Test>						= title derived from TS 36.101.

<Release>	= optional identifier showing the release applicability of the test. To be included only when there is already a test with the same title for earlier releases.

Examples:

1.2.3	Virtual test

1.2.3_H	Virtual test for SCE-L1

1.2.3_H.1	Virtual test for SCE-L1(Single Carrier)

1.2.3_H.2	Virtual test for SCE-L1(2DL CA)

1.2.3_H.3	Virtual test for SCE-L1(3DL CA)

### I.2.10.3	SCE-L1 CSI tests (Chapter 9)

SCE-L1 Performance minimum requirements in 3GPP TS 36.101 [2] are specified in separate clauses from the legacy requirements. The set of SCE-L1 test points may have a wide coverage in terms of Annex I.1 bullet 3)b) and c) etc. In such cases in TS 36.521-1 are specified several separate "individual tests" for SCE-L1.

Rule:

Following tests and clauses are specified in TS 36.521-1:

Test (group) clause:

<x.x.x>_H	<Test> for SCE-L1

Individual tests:

<x.x.x>_H.<y>	<Test> for SCE-L1 (<Release>)

where:

<x.x.x>						= number derived from TS 36.101.

<y>							= optional incrementing clause number (Arabic numeral) (if many individual tests).

<Test>						= title derived from TS 36.101.

<Release>	= optional identifier showing the release applicability of the test. To be included only when there is already a test with the same title for earlier releases.

Examples:

1.2.3	Virtual test

1.2.3_H	Virtual test for SCE-L1

## I.2.11	Tests for ProSe

### I.2.11.1	ProSe Tx tests (Chapter 6)

ProSe specific Transmitter minimum requirements in 3GPP TS 36.101 [2] are specified in separate clauses from the legacy requirements, bearing the suffix "D" in the number and "for ProSe" in the title. The set of ProSe test points may have a wide coverage in terms of Annex I.1 bullet 3)b) and c) with respect of type of ProSe operation and UE capability (Discovery / Communication) etc. In such cases in TS 36.521-1 are specified several separate "individual tests" for ProSe.

Rule:

Following tests and clauses are specified in TS 36.521-1:

<x.x.x>D		<Test> for ProSe

<x.x.x>D<.y><_z>	<Test> for ProSe <Mode> (<Release>)

where:

<x.x.x>	= number derived from TS 36.101 without suffix "D"

<.y>	= incrementing clause number (Arabic numeral).

<_z>	= optional incrementing clause number (Arabic numeral) with preceding underscore. To be applied only when there is already a test with the same title for earlier releases.

<Test>	= title derived from TS 36.101 without "for ProSe"

<Mode>	= mandatory identifier {Direct Discovery; Direct Communication}

<Release>	= optional identifier showing the release applicability of the test. To be included only when there is already a test with the same title for earlier releases.

Examples:

1.2.3D		Virtual test for ProSe

1.2.3D.1		Virtual test for ProSe Direct Discovery

1.2.3D.2		Virtual test for ProSe Direct Communication

1.2.3D.2_1		Virtual test for ProSe Direct Communication (Release 13 and forward)

### I.2.11.2	ProSe Rx tests (Chapter 7)

Same as for Transmitter tests (Chapter 6) in Annex I.2.11.1.

### I.2.11.3	ProSe Performance tests (Chapter 8)

ProSe Performance minimum requirements in 3GPP TS 36.101 [2] are already specified in separate clauses from the legacy requirements.

Rule:

Following tests and clauses are specified in TS 36.521-1:

FFS

### I.2.11.4	ProSe CSI tests (Chapter 9)

Same as for Performance tests (Chapter 8) in Annex I.2.11.3.

## I.2.12	Tests for 4 Rx antenna ports

### I.2.12.1	4 Rx antenna ports Rx tests (Chapter 7)

4 Rx antenna ports minimum requirements in 3GPP TS 36.101 [2] are specified in the same clauses as the legacy requirements. The reference sensitivity requirements in section 7.3 are separate from the old test, but the rest of the Receiver tests have the same requirements as the old tests (tested at a level based on REFSENS). But since the test points are different, new variant test cases are still needed.

In TS 36.521-1 separate 2Rx and 4Rx tests are specified for Rel8 and Rel9 tests, but 4Rx is added to the existing tests starting from Rel10 test cases.

Rule:

Following tests and clauses are specified in TS 36.521-1:

Test group before introducing 4Rx:

<x.x.x>	<Test> (Rel8 or Rel9)

<y.y.y>	<Test> (Rel10 or later)

Test group after introducing 4Rx:

<x.x.x>	<Test> for 2Rx

<x.x.x>_1	<Test> for 4Rx

<y.y.y>	<Test> for 2Rx and 4Rx

where:

<x.x.x>	= number of the corresponding legacy test.

<y>	= incrementing clause number (Arabic numeral)

<Test>	= title of the corresponding legacy test

(Release)	= Release applicability (applicable from), which is not part of the title

Examples:

1.2.3		Virtual test (Rel8)

1.2.3_1	Virtual test with 4 Rx antenna ports (Rel10)

1,2,4	Virtual test (Rel10)

### I.2.12.2	4 Rx antenna ports Performance tests (Chapter 8)

4 Rx antenna ports Performance minimum requirements in 3GPP TS 36.101 [2] are specified in separate clauses from the legacy requirements. Separate test cases for 4Rx antenna ports are needed in TS 36.521-1 with test case numbering following TS 36.101[2] numbering,

### I.2.12.3	4 Rx antenna ports CSI tests (Chapter 9)

TBD

Annex J (normative):
Modified MPR behaviour

Editor’s note: The modifiedMPRbehavior is given in the Annex H.1 of TS 36.101.

# J.1	Indication of modified MPR behaviour

This annex contains the definitions of the bits in the field modifiedMPRbehavior indicated in the IE UE Radio Access Capability [5] by a UE supporting an MPR or A-MPR modified in a later release of this specification.

Table J.1-1: Definitions of the bits in the field modifiedMPRbehavior (Rel-10 and Rel-11)

| Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- |
| 0 (leftmost bit) | The MPR for intra-band contiguous carrier aggregation bandwidth class C with non-contiguous resource allocation specified in Clause 6.2.3A in version 12.5.0 of TS36.101[2] | This bit can be set to 1 by a UE supporting intra-band contiguous CA bandwidth class C |
| 1 | The A-MPR associated with NS_05 for Band 1 in Clause 6.2.4 in version 12.10.0 of TS36.101[2] | This bit can be set to 1 by a UE supporting A-MPR associated to NS_05 for Band 1 |

Table J.1-2: Definitions of the bits in the field modifiedMPRbehavior (Rel-12 and Rel-13)

| Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- |
| 0 (leftmost bit) | The MPR for intra-band contiguous carrier aggregation bandwidth class C with non-contiguous resource allocation specified in Clause 6.2.3A in version 12.5.0 of TS36.101 [2] | This bit shall be set to 1 by a UE supporting intra-band contiguous CA bandwidth class C |
| 1 | The A-MPR associated with NS_05 for Band 1 in Clause 6.2.4 in version 12.10.0 of TS36.101[2] | This bit shall be set to 1 by a UE supporting A-MPR associated to NS_05 for Band 1 |
| 2 | The A-MPR associated with NS_04 for Band 41 in Table 6.2.4-4 in version 14.2.0 of this specification. | This bit can be set to 1 by a power class 3 UE supporting A-MPR associated to NS_04 for Band 41. |

Table J.1-3: Definitions of the bits in the field modifiedMPRbehavior (Rel-14 and later)

| Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- |
| 0 (leftmost bit) | The MPR for intra-band contiguous carrier aggregation bandwidth class C with non-contiguous resource allocation specified in Clause 6.2.3A in version 12.5.0 of TS36.101 [2] | This bit shall be set to 1 by a UE supporting intra-band contiguous CA bandwidth class C |
| 1 | The A-MPR associated with NS_05 for Band 1 in Clause 6.2.4 in version 12.10.0 of TS36.101[2] | This bit shall be set to 1 by a UE supporting A-MPR associated to NS_05 for Band 1 |
| 2 | The A-MPR associated with NS_04 for Band 41 in Table 6.2.4-4 in version 14.2.0 of this specification. | This bit shall be set to 1 by a power class 3 UE supporting A-MPR associated to NS_04 for Band 41. |

Annex K (normative):
NB-IoT Test Frequencies

# K.1	NB-IoT Test frequencies for TRx Tests

Testing frequencies for all NB-IoT TRx test cases (sections 6 and 7 in TS 36.521-1) should be selected across the bands UE supports from any of the following subclauses indicated by each test case.

For transmitter test cases (section 6 in TS 36.521-1), UL frequencies should be considered on the test frequency selection algorithm described above, while DL frequencies should be used for receiver test cases (section 7 in TS 36.521-1).

Refer to TS 36.508 [7] section 8.1.3.1 for testing frequencies associated to each frequency band and each operation mode.

## K.1.1	Test frequencies selection criteria 1

Among all frequency bands ranges below 1GHz supported by the UE, testing points are defined as:

-	@ low range of the lowest supported band

-	@ high range of the highest supported band

If the UE does not support any frequency band below 1GHz, these test points are not applicable.

Among all frequency bands ranges above 1GHz supported by the UE, testing points are defined as:

-	@ low range of the lowest supported band

-	@ high range of the highest supported band

If the UE does not support any frequency band above 1GHz, these test points are not applicable.

## K.1.2	Test frequencies selection criteria 2

Among all frequency bands ranges below 1GHz supported by the UE, testing points are defined as:

-	@ low range of the lowest supported band

-	@ high range of the highest supported band

-	All mid range of all supported mid bands (supported bands between lowest supported band and highest supported band below 1GHz)

If the UE does not support any frequency band below 1GHz, these test points are not applicable.

Among all frequency bands ranges above 1GHz supported by the UE, testing points are defined as:

-	@ low range of the lowest supported band

-	@ high range of the highest supported band

-	All mid range of all supported mid bands (supported bands between lowest supported band and highest supported band above 1GHz)

If the UE does not support any frequency band above 1GHz, these test points are not applicable.

## K.1.3	Test frequencies selection criteria 3

Among all frequency bands ranges supported by the UE, testing points are defined as:

-	@ low range of each supported band

-	@ high range of each supported band

# K.2	NB-IoT Test frequencies for Demodulation Tests

Testing frequencies for all NB-IOT demodulation (Section 8 and 9 in TS 36.521-1) should be selected across the bands UE supports from any of the following subclauses indicated by each test case.

For NB-IOT demodulation (section 8 and 9 in TS 36.521-1) DL frequencies should be used from the frequency selection algorithm described below.

Refer to section 8.1.3.1 in TS 36.508 [7] for testing frequencies associated to each frequency band and each operation mode.

### K.2.1	Test frequencies selection criteria 1

Among all frequency bands ranges supported by the UE, testing points are defined as:

-	@ mid range of any one of the NB1 and NB2 UE supported band

Annex L (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2007-08 | RAN5 #36 | R5-072185 |  |  | Skeleton proposed for RAN5#36Athens |  | 0.0.1 |
| 2007-08 | RAN5 #36 | R5-072419 |  |  | Update the skeleton base on R4-071234_TR36.803.0.4.0.doc | 0.0.1 | 0.0.2 |
| 2007-08 | RAN5 #36 | R5-072424 |  |  | Update with editorial changes | 0.0.2 | 0.0.3 |
| 2007-11 | RAN5 #37 | R5-073043 |  |  | Update document with some info as following: Section 5: Frequency band information Section 6.2: Maximum output power Section 6.5: Output RF spectrum emissions Section 6.5.1: Occupied bandwidth Section 6.5.2: Out of band emission Section 6.5.3: Spurious emissions | 0.0.3 | 0.0.4 |
| 2007-11 | RAN5 #37 | R5-073360 |  |  | Editorial change to split MOP and UE Power classes | 0.0.4 | 0.0.5 |
| 2008-03 | RAN5 #38 | R5-080069 |  |  | Editorial changes to sync up with 36.101 v1.0.0 as much as feasible for the moment: Update definitions, symbols and abbreviations Update frequency bands, channel bandwidth, channel numbers information. Restructure document to move “frequency error” sub-section inside Transmit signal quality. Add “additional spectrum Emission Mask” sub-test (mask A,B,C) section to address the regulatory requirements that are not met with the general mask (OOB and spurious emission). Add “Additional ACLR requirements” to address additional requirements that the network might indicate to the UE via signalling for a specific deployment scenario (in terms of additional requirements for UTRA/ACLR2 Restructure “Spurious Emission” to indicate we need to have 3 test cases to address: “E-UTRA Spurious Emission” requirements , “Spurious Emission band UE co-existence” requirements, and “Additional spurious emissions” requirements Separate wide band and narrow band intermodulation in the intermodulation characteristics | 0.0.5 | 0.0.6 |
| 2008-03 | RAN5 #38 | R5-080408 |  |  | LTE Reference Sensitivity test Text proposal |  | 0.0.7 |
| 2008-03 | RAN5 #38 | R5-080409 |  |  | LTE Maximum Rx input level test Text proposal |  | 0.0.7 |
| 2008-03 | RAN5 #38 | R5-080410 |  |  | LTE Adjacent Channel Selectivity test Text proposal |  | 0.0.7 |
| 2008-03 | RAN5 #38 | R5-080064 |  |  | LTE RF Receiver tests, General section Text proposal |  | 0.0.7 |
| 2008-03 | RAN5 #38 | R5-080412 |  |  | LTE RF: transmission modulation initial EVM test proposal |  | 0.0.7 |
| 2008-03 | RAN5 Workshop-UE LTE Test (9-11 April) | R5w0800027 |  |  | Modify styles and formats of tables and others according to drafting rules. Add some definitions and abbreviations Modified section 6.2 structure to be aligned with 36.101 v8.1.0 Modify tables of requirements to remove 1.6 MHz and 3.2MHz channel bandwidth according to new requirements 36.101 v8.1.0 |  | 0.0.9 |
| 2008-03 | RAN5 Workshop-UE LTE Test (9-11 April) | R5w0800028 |  |  | Following TPs have been included: R5w080013r1 R5w080014r1 R5w080008r2 R5w080009r2 R5w080040r1 R5w080015r1 R5w080016r1 R5w080017r1 R5w080018r2 | 0.0.9 | 0.1.0 |
| 2008-05 | RAN5#39 | R5-081046 |  |  | 36-521-1 alignment of measurement state for test cases | 0.1.0 | 0.1.1 |
| 2008-05 | RAN5#39 | R5-081042 |  |  | Following approved TPs have been included: R5-081040 36.521-1 after April LTE-RF workshop R5-081415 36-521-1 alignment of measurement state for test cases – also the measurement state for each test cases has been updated according to R5-081404 R5-081416 Cover for LTE E-UTRAN RRC_IDLE State Mobility text proposal R5-081417 Cover for LTE E-UTRAN RRC_CONNECTED State Mobility text proposal R5-081404 LTE Rx Intermodulation test case text proposal R5-081409 Annex structure for Measurement uncertainty & Test Tools R5-081405 Text Proposal for TS 36.521-1 TC7.6 Blocking Characteristics R5-081406 Text Proposal for TS 36.521-1 TC7.7 Spurious Response R5-081403 Text Proposal for TS 36.521-1 TC7.9 Spurious Emissions R5-081410 Uncertainties and Test Tools for subset of UE tests R5-081331 Clarification of diversity characteristics section for multiple UE antennas R5-081335 36-521-1 update of nominal and additional channel bandwidths | 0.1.1 | 0.2.0 |
| 2008-06 | RAN5 #39bis | R5-082029 |  |  | Following approved TPs have been included: R5-082129: Restructure of TS 36.521-1 and RRM proposal (Split of RRM from 36.521-1 v0.2.0 in its own specification 36.521-3.) R5-082166: Text Proposal for Annex C Downlink Physical Channels R5-082130: Text Proposal for Chan bandwidths in TS 36.521-1 R5-082155: Text Proposal for LTE Tx Minimum Output Power R5-082027: Text Proposal for Occupied bandwidth in TS 36.521-1 R5-082171: Text Proposal for LTE Adjacent Channel Leakage power Ratio R5-082134: Text Proposal for LTE Tx Spurious Emissions R5-082135: Text Proposal for LTE UE Maximum Output Power R5-082136: Text Proposal for LTE Spectrum Emission Mask R5-082138: UE Spurious Emissions Measurement uncertainty & Test Tolerances R5-082169: LTE Spectrum Emission Mask test uncertainties and TTs R5-082151: LTE UE Max Power and ACLR tests uncertainties and TTs R5-082152: Text proposal for LTE Transmit OFF Power R5-082153: LTE UE Max Rx Input and ACS test cases update R5-082082: LTE Rx Intermodulation test case uncertainties and TTs R5-082093: Text Proposal for TS 36.521-1 TC7.6 Blocking Characteristics R5-082154: Text Proposal for TS 36.521-1 TC7.7 Spurious Response R5-082167: OBW Measurement uncertainty & Test Tolerances R5-082158: Cover for LTE Performance Requirement text proposal R5-082159: Text Proposal for LTE Demodulation of PCFICH/PDCCH and PHICH R5-082156: Text proposal for LTE Tx Minimum Output Power Uncertainty R5-082157: Text proposal for LTE Tx Minimum Output Power Tolerance R5-082164: Statistical testing of receiver characteristics R5-082170: Cover for LTE Propagation Conditions Text Proposal Editorial changes to align tables and figures numbering with R5-082025 | 0.2.0 | 0.3.0 |
| 2008-08 | RAN5 #40 | R5-083163 |  |  | Following approved TPs have been included: R5-083804: LTE Demodulation Performance text proposal R5-083159: LTE-RF Occupied bandwidth test case / measurement uncertainty and TT text proposal R5-083160: Transmission OFF power: TP, measurement uncertainty and test tolerances proposal R5-083805: Frequency Error test case / measurement uncertainty and TT test proposal R5-083162: Propagation conditions correction text proposal R5-083220:Text Proposal for LTE Tx Minimum Output Power R5-083806: TP of section 8 for E-UTRAN TDD in 36.521-1 R5-083344: Test Tolerance and System uncertainty for OBW test R5-083848:Test Tolerance and System uncertainty for Reference sensitivity test R5-083840: Test Tolerances for Spectrum Emission Mask R5-083808: Reference Measurement Channel for LTE UE Receiver tests R5-083350: Test Tolerance and System uncertainty for Blocking and Spurious response R5-083366: Text Proposal for LTE Reporting of CQI/PMI R5-083810: LTE PBCH Demodulation Performance Requirements R5-083482: LTE-RF TP for Test Case 7.6 Blocking Characteristics R5-083809: LTE-RF TP for Test Case 7.7 Spurious Response R5-083484: LTE-RF TP for Test Case 7.9 Spurious Emissions R5-083811: Annex E Global In-Channel TX-Test R5-083163: TS 36.521-1 after RAN5#40 | 0.3.0 | 1.0.0 |
| 2008-10 | RAN5 #40Bis | R5-084072 |  |  | Following approved TPs have been included: R5-084072 TS 36.521-1 after RAN5#40Bis R5-084300 LTE-RF TP for Definitions Symbols and Abbreviations R5-084304 LTE-RF-TP for general section R5-084036 Test Tolerances for additional SEM R5-084303 LTE-RF TP for Channel bandwidths and frequency range R5-084305 LTE-RF TP for new Absolute Power Tolerance test case R5-084067 LTE-RF TP for Transmission OFF test case R5-084318 LTE-RF TP for Transmission Modulation test cases R5-084069 LTE-RF Investigation of E-UTRA-TDD Frequency Error test case applicability R5-084319 LTE-RF TP for Frequency Error test case R5-084309 Text Proposal for LTE Tx Spurious Emissions R5-084111 Text Proposal for LTE Adjacent Channel Leakage power Ratio R5-084320 Text Proposal for LTE Additional Spectrum Emission Mask R5-084310 Test Tolerances for additional spurious emission R5-084311 Text Proposal for Occupied bandwidth R5-084321 Text Proposal for LTE Spectrum Emission Mask R5-084060 Modification to section 7.2 Diversity characteristics R5-084312 References in 36.521-1 tests initial conditions R5-084148 Update of Reference Measurement Channel for LTE UE Rx tests R5-084167 LTE-RF TP for TC7.9 Spurious Emissions R5-084075 LTE DL Reference Measurement Channel for PDSCH (FDD) text proposal R5-084077 LTE Measurement of Performance Requirements text proposal R5-084313 LTE Demodulation of PDSCH Test Requirements text proposal R5-084147 Specification of DL propagation conditions for LTE UE tests R5-084315 Text Proposal for LTE Demodulation of PCFICH/PDCCH R5-084323 Text Proposal for Annex E Global In-Channel | 1.0.0 | 1.1.0 |
| 2008-12 | RAN#42 | RP-080863 |  |  | Approval of version 2.0.0 at RAN#42, then put to version 8.0.0. | 2.0.0 | 8.0.0 |
| 2008-01 |  |  |  |  | Editorial corrections. | 8.0.0 | 8.0.1 |
| 2009-03 | RAN#43 | R5-086011 | 0001 | - | TP for In-band emissions | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-086012 | 0002 | - | TP for Spectrum flatness | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-086013 | 0003 | - | TP for IQ-component | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-086064 | 0004 | - | LTE-RF: UE max output power | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-086093 | 0005 | - | Clarification of measurement period in minimum output power test procedure | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-086094 | 0006 | - | Clarification of measurement period in transmit OFF power test procedure | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-086120 | 0007 | - | Update of Max.input level test | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-086125 | 0008 | - | Addition of UL Reference Measurement Channels in Annex A2 | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-086160 | 0009 | - | correction for Maximum Power Reduction (MPR) | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-086167 | 0010 | - | LTE-RF: TDD applicability and CR for Blocking Characteristics and Spurious Response | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-086168 | 0011 | - | LTE-RF: TDD applicability and CR for Spurious Emissions | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-086239 | 0012 | - | Update of Symbols | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-086401 | 0013 | - | LTE-RF: TX-RX channel freq separation | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-086405 | 0014 | - | Update of 6.7 Transmit intermodulation test | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-086406 | 0015 | - | Update of initial conditions for Tx and Rx test cases | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-086408 | 0016 | - | Update of Adjacent Channel Leakage power Ratio | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-086409 | 0017 | - | Removal of [ ] from Clause 7 Receiver Characteristics | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-086413 | 0018 | - | Updates to Demodulation of PCFICH/PDCCH test case | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-086414 | 0019 | - | Text proposal for Reporting of Channel State Information | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-086415 | 0020 | - | Correction of RS_EPRE powers for default DL signal levels | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-086416 | 0021 | - | Update of DL Reference Measurement Channels in Annex A3 | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-086417 | 0022 | - | Update to Annex E | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-086425 | 0023 | - | Update of General text in clause 6 | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-086426 | 0024 | - | Clarification of measurement bandwidth in spectrum emission mask test | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-086428 | 0025 | - | Demodulation of TDD PHICH test requirements text proposal | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-086429 | 0026 | - | Demodulation of TDD PCFICH/PDCCH test requirements text proposal | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-090306 | 0027 | - | New Annex H for Uplink Physical Channels | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-090308 | 0028 | - | Text proposal for Reporting of Channel State Information | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-090403 | 0029 | - | CR to 36.521-1: Update of Spurious Emissions test cases | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-090404 | 0030 | - | CR to 36.521-1: Update of ACLR test case | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-090443 | 0031 | - | LTE-RF: Correction to 36.521-1 Frequency error test case | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-090488 | 0032 | - | LTE TDD applicability for Transmit intermodulation test case | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-091002 | 0033 | - | LTE Demodulation of PDSCH Test Requirements text proposal | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-091004 | 0034 | - | LTE-RF: CR for UE max power test case | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-091007 | 0035 | - | LTE-RF: TDD Applicability and CR for Spectrum Emission Mask and Additional Spectrum Emission Mask | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-091008 | 0036 | - | LTE-RF Investigation of E-UTRA-TDD for Occupied bandwidth test case applicability | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-091009 | 0037 | - | LTE-RF: Investigation of E-UTRA-TDD for Adjacent Channel Leakage power Ratio test case applicability | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-091011 | 0038 | - | LTE-RF: TDD applicability and CR for Maximum Input Level | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-091012 | 0039 | - | LTE-RF: TDD applicability and CR for Adjacent Channel Selectivity (ACS) | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-091017 | 0040 | - | Removal of Rx Narrowband Intermod 7.8.2 | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-091019 | 0041 | - | Relocation of 36.521-1 Annex C DL mapping | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-091020 | 0042 | - | Removal of "Out-of-synchronization handling of output power" heading | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-091023 | 0043 | - | Test requirements of TDD PDSCH demodulation performance with user-specific reference symbols | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-091024 | 0044 | - | CR to 36.521-1: Update of Annex F.3.2 Measurement of transmitter | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-091025 | 0045 | - | CR to 36.521-1: Update of SEM and Additional SEM test cases | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-091077 | 0046 | - | CR to 36.521-1: Addition of test combinations for test cases with MPR application | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-091082 | 0047 | - | Spurious emission requirements on PHS band including the future plan in Japan | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-091101 | 0048 | - | LTE-RF: CR for MPR test case | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-091106 | 0049 | - | Update of Reference sensitivity test in 7.3 | 8.0.1 | 8.1.0 |
| 2009-03 | RAN#43 | R5-091111 | 0050 | 1 | Update of initial conditions for Rx tests | 8.0.1 | 8.1.0 |
| 2009-05 | RAN#44 | R5-092144 | 0051 | - | LTE-RF: Resubmission of R5-086424 UE output power dynamics 36.521-1 v8.1.0 (re-submit no changes) | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092146 | 0052 | - | LTE-RF: CR for UE configured UE transmitted output power test case (re-submit no changes) | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092147 | 0053 | - | LTE-RF: CR for UE minimum output power test case (re-submit no change) | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092149 | 0054 | - | LTE-RF: CR for Power Control Absolute power tolerance test case (re-submit no changes) | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092150 | 0055 | - | LTE-RF: CR for Power Control Relative power tolerance test case (re-submit no changes) | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092151 | 0056 | - | LTE-RF: New test case for Aggregate power control tolerance (re-submit no changes) | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092263 | 0057 | - | Text proposal for Reporting of Channel State Information | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092264 | 0058 | - | Propagation conditions for CQI tests | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092265 | 0059 | - | Correction to Demodulation of PDCCH/PCFICH test cases | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092273 | 0060 | - | Mapping of downlink physical channels for TDD | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092277 | 0061 | - | Annex A RMC updates | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092369 | 0062 | - | Update of A.3.4.3 for RMC with UE-specific RS | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092372 | 0063 | - | Maintenance on Initial configurations for Perf TCs | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092436 | 0064 | - | CR to 36.521-1: Update of ACLR test case | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092442 | 0065 | - | CR to 36.521-1: Update of Spurious Emissions test case | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092467 | 0066 | - | LTE-RF: Transmit OFF Power update | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092473 | 0067 | - | LTE_RF - Update on TC 7.7 Spurious Response (re-submit with no changes) | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092474 | 0068 | - | LTE_RF - Update on TC 7.9 Spurious Emissions (re-submit with no changes) | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092527 | 0069 | - | Update of TDD PDSCH test cases | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092602 | 0070 | - | LTE-RF: CR for Maximum Power Reduction test case (re-submit no changes) | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092603 | 0071 | - | TP for Demodulation of TDD PDCCH/PCFICH | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092605 | 0072 | - | Mapping of uplink physical channels for FDD | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092606 | 0073 | - | Update of Annex C | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092607 | 0074 | - | CR to 36.521-1: Update of test parameters for Demodulation of PDSCH (FDD) tests | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092614 | 0075 | - | Update of SEM test case | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092642 | 0076 | - | Update of transmit quality test cases | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092643 | 0077 | - | Text proposal for TDD part of CQI Reporting under Fading conditions | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092644 | 0078 | - | Text proposal for TDD part of CQI Reporting under AWGN conditions | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092645 | 0079 | - | LTE-RF: Update of Additional Spectrum Emission mask Test case with TDD Uplink Test configuration | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092649 | 0080 | - | LTE-RF: CR for TDD DL RMC to be used in TX test cases | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092653 | 0081 | - | LTE-RF: CR for Additional Maximum Power Reduction test case | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092661 | 0082 | - | RMC update for PDCCH/PCFICH performance requirement | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | RP-090444 | 1161 | - | Test frequencies for Additional Spurious Emission test case | 8.6.0 | 8.7.0 |
| 2009-05 | RAN#44 | R5-092366 | 0084 | - | Update of 7.3.1 | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092440 | 0085 | - | LTE-RF: CR for UE max output power test case | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092472 | 0086 | - | LTE_RF - Update on TC 7.6 Blocking Characteristics (re-submit with changes) | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092636 | 0087 | - | CR to 36.521-1 Addition of frequencies for band 18 and band 19 | 8.1.0 | 8.2.0 |
| 2009-05 | RAN#44 | R5-092652 | 0088 | 2 | Improved stability of TC 7.8.5 Power Control in the DL from F-DPCH to HSUPA TC 5.2D and 5.13.2B | 8.1.0 | 8.2.0 |
| - | - | - | - | - | Editorial corrections | 8.2.0 | 8.2.1 |
| 2009-09 | RAN#45 | R5-094032 | 0089 | - | Correction CR to 36.521-1: Update of Requirements for Demodulation of PDSCH (FDD) tests | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094034 | 0090 | - | Correction CR to 36.521-1: Update of General Requirements for Demodulation tests | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094214 | 0091 | - | Update of In-band emissions | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094215 | 0092 | - | TDD Initial downlink channel setting | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094216 | 0093 | - | Correction to Annex B | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094248 | 0094 | - | CR to 36.521-1: Update to ACLR test case | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094250 | 0095 | - | CR to 36.521-1: Update to UE max output power test case | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094281 | 0096 | - | Mapping of uplink physical channels for TDD | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094282 | 0097 | - | LTE-RF: CR for notes in TDD DL RMC to be used in TX test cases | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094283 | 0098 | - | LTE-RF: message update to keep Tx power constant for some Rx test cases | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094313 | 0099 | - | LTE-RF: CR to test case for Aggregate power control tolerance | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094317 | 0100 | - | LTE-RF: CR for UE minimum output power test case for TDD | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094318 | 0101 | - | LTE-RF: CR for Power Control Relative power tolerance test case | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094319 | 0102 | - | In band emission for non-allocated RB | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094320 | 0103 | - | LTE RF: correction for subclause 6.6.2.2.5 (A-SEM) supported band list | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094362 | 0104 | - | Correction of RMCs (36.521 Annex A) | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094363 | 0105 | - | Usage of the Global In-Channels TX-Test across different Signal Quality tests. | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094365 | 0106 | - | LTE TX: 1to2 RX antenna | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094367 | 0107 | - | Correction to 6.6.2.2 Additional Spectrum Emission Mask | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094370 | 0108 | - | Correction to 6.6.2.3 ACLR | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094371 | 0109 | - | Correction to 6.7 TX Intermodulation | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094374 | 0110 | - | Correction to 7.6.1 In-Band Blocking | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094375 | 0111 | - | UE category (36.521 clause 8) | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094378 | 0112 | - | Completion of Global in-Channel TX-Test (36.521 Annex E) | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094379 | 0113 | - | Completion of Global in-Channel TX-Test with PRACH (36.521 Annex E) | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094380 | 0114 | - | Completion of Statistical testing (36.521 Annex G) | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094385 | 0115 | - | Correction to Annex D.2 Interference signals | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094439 | 0116 | - | Update for ACS | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094661 | 0117 | - | LTE RF - Core update on TC7.6.2 Out-of-band Blocking | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094663 | 0118 | - | LTE RF - Symbols Update on UL transmission configurations | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094665 | 0119 | - | LTE RF - Clarification for Test Configurations in General Section | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094668 | 0120 | - | LTE RF - Applicability of 6.2.3 MPR | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094671 | 0121 | - | LTE RF - Verification of UE Output Power in Out of Band Emission tests | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094684 | 0122 | - | CR to 36.521-1: Update to UE max output power test case | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094686 | 0123 | - | LTE-RF CR to 36.521-1: Update the E-UTRA channel numbers | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094687 | 0124 | - | LTE-RF: CR for UE maximum power reduction(MPR) test case | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094699 | 0125 | - | Update to SEM and spurious emissions TC | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094706 | 0126 | - | Resubmission-Update to the Requirements for frequency-selective fading test | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094717 | 0127 | - | Update of SEM | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094718 | 0128 | - | Update of initial conditions with Annex references | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094721 | 0129 | - | Update of 6.7 Tx Inter Mod | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094725 | 0130 | - | Correction to E-UTRA channel numbers for Band 2 | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094726 | 0131 | - | Correction to Tx spurious emissions | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094757 | 0132 | - | Update of TDD PHICH test cases | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094874 | 0133 | - | Correction to Demodulation of PDCCH/PCFICH test cases | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094902 | 0134 | - | Addition of 15 MHz and 20 MHz bandwidths and corresponding sensitivity requirements into band 38 | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094903 | 0135 | - | Correction CR to 36.521-1: Update of Transmitter tests network signalled parameter value | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094905 | 0136 | - | Update of TDD PDSCH test cases | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094908 | 0137 | - | LTE-RF: CR for Power Control Absolute power tolerance test case | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094909 | 0138 | - | Update to Output Power dynamics test cases | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094913 | 0139 | - | Clarification for downlink signal setting in RX tests | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094914 | 0140 | - | UL RB allocation for receiver tests | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094915 | 0141 | - | Update of TDD PCFICH/PDCCH test cases | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094921 | 0142 | - | Correction to CQI performance test case | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094922 | 0143 | - | Test description for CQI test cases under AWGN conditions | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094923 | 0144 | - | Resubmission - Requirements for PMI reporting ( Single and Multiple PMI) | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094966 | 0145 | - | CR to 36.521-1: Addition of A-MPR for band 19 | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094976 | 0146 | - | Without loop back: 6.2.2 UE maximum output power | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094977 | 0147 | - | Without loop back: 6.3.2 Minimum output power | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094979 | 0148 | - | LTE-RF: CR for UE configured UE transmitted output power test case | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094980 | 0149 | - | CR to 36.521-1: Definition of Maximum Power state in TX/RX test cases | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094982 | 0150 | 1 | Correction of Tx general description | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094986 | 0151 | - | Update of 6.6.1OBW | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094989 | 0152 | - | Correction to 1PRB tests in Demodulation of PDSCH | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094995 | 0153 | - | Correction CR to 36.521-1: Update of Requirements for Additional Maximum Power Reduction (A-MPR) test | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094996 | 0154 | - | Correction to Demodulation of PHICH test cases | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-094997 | 0155 | - | EVM TC update | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-095300 | 0156 | - | LTE-RF: test description update | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-095301 | 0157 | - | Correction CR to 36.521-1: Addition of measurement uncertainty and test tolerances for A-MPR | 8.2.1 | 8.3.0 |
| 2009-09 | RAN#45 | R5-095304 | 0158 | - | Sorting out Demodulation of PDSCH for FDD | 8.2.1 | 8.3.0 |
| 2009-09 | - | - | - | - | TOC update and Annexes' titles formattings | 8.3.0 | 8.3.1 |
| 2009-12 | RAN#46 | R5-095515 | 0159 | - | Correction CR to 36.521-1: Additional Spectrum Emission Mask test need to be updated to include the network signalled value "NS_07ö message contents exceptions | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-095589 | 0160 | - | Update for test period description in the general section | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-095657 | 0161 | - | LTE-RF: CR for Power Control Absolute power tolerance test case | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-095661 | 0162 | - | LTE-RF: CR for UE minimum output power test case | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-095735 | 0163 | - | Corrections to Annex A.4 | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-095766 | 0164 | - | LTE-RF: CR for In band emission for non-allocated RB | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-095790 | 0165 | - | Completion of Statistical testing (36.521 Annex G) | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-095791 | 0166 | - | Corrections to Annex E | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096058 | 0167 | - | Removal of [ ] from 7.6.1, 7.8.1, and 7.5 of Annex F3.3 | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096096 | 0168 | - | Update on 8.2.1 | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096105 | 0169 | - | LTE RF: Symbols Update on Configured UE Transmitted Power | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096204 | 0170 | - | LTE-RF: CR to Transmission signal quality | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096208 | 0171 | - | LTE-RF: CR for Power Control Relative power tolerance test case | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096210 | 0172 | - | LTE-RF: CR to ON/OFF Time mask test cases | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096211 | 0173 | - | Measurement period for TX-Tests | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096213 | 0174 | - | CR to 36.521-1: Update to Spurious Emissions test cases | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096214 | 0175 | - | CR to 36.521-1: Update to ACLR test case | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096219 | 0176 | - | LTE-RF: CR for UE configured UE transmitted output power test case | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096222 | 0177 | - | Test description for CQI test cases under AWGN conditions | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096223 | 0178 | - | LTE RF: Blocking Characteristics update | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096224 | 0179 | - | LTE RF: Spurious Response Update | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096228 | 0180 | - | LTE-RF: CR for MPR test case | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096229 | 0204 | 2 | CR to 36.521-1: Update to A-MPR test case | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096230 | 0181 | - | LTE RF: Applicability of 6.2.4 A-MPR | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096231 | 0182 | - | Correction to Demodulation of PHICH test cases | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096235 | 0183 | - | Introduction of CQI reporting test with frequency-selective interference | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096239 | 0184 | - | Update to the test procedure and message contents of TDD PMI reporting test cases | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096240 | 0205 | - | CR to 36.521-1: Update to Derivation of Test Requirements for A-MPR | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096241 | 0185 | - | Measurement uncertainties and Test Tolerances for transmit quality test cases | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096242 | 0186 | - | Update for 36.521-1 Annex A | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096289 | 0187 | - | CR on 36.521-1, 'Introduction of clause 8.2.1.1 test case uncertainties and Test Tolerances' | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096306 | 0188 | - | Update to the test procedure of SEM test cases of 36.521-1 | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096311 | 0189 | - | Update of 6.6.1 OBW | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096312 | 0190 | - | Correction to SEM | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096313 | 0191 | - | Update of 6.7 Transmit intermodulation | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096315 | 0192 | - | CR to 36.521-1: Update to UE max output power test case | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096316 | 0193 | - | CR to 36.521-1: Update to Additional Spurious Emissions test case | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096317 | 0194 | - | CR to TDD PHICH demodulation test cases | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096318 | 0195 | - | Correction to FDD PMI reporting test cases | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096320 | 0196 | - | Tx power range and core update for Receiver tests | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096322 | 0197 | - | Update on 7.4, 7.5, and 7.8.1 | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096323 | 0198 | - | Introduction of RI reporting test | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096333 | 0199 | - | Update to 6.5 Transmit signal quality test cases | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096334 | 0200 | - | LTE-RF: CR for Aggregate power control tolerance test case | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096335 | 0201 | - | Correction CR to 36.521-1: Update for Demodulation of PDSCH (FDD) tests to correct CR merges results from RAN5#44 | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096336 | 0206 | 1 | Update TDD PDSCH test cases | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096338 | 0202 | - | Number of used HARQ processes in DL Performance tests | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096342 | 0207 | 2 | Minimum test time for performance tests | 8.3.1 | 8.4.0 |
| 2009-12 | RAN#46 | R5-096718 | 0203 | - | LTE RF: A-SEM update and A-MPR verification | 8.3.1 | 8.4.0 |
| 2010-03 | RAN#47 | R5-100353 | 0208 | - | LTE-RF CR to 36.521-1:TIME MASK test case updated | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100354 | 0209 | - | LTE-RF: CR for A-MPR notation in NS_07 | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100403 | 0210 | - | LTE-RF: CR for Tx Intermodulation test case | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100404 | 0211 | - | LTE-RF: CR for OBW measurement period alignment | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100408 | 0212 | - | Reporting mode, Reporting Interval and Editorial corrections for demodulation | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100456 | 0213 | - | Misc update on MAC padding in Rx and performance sections | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100566 | 0214 | - | Missing Test limits in 36.521-1 Annex G | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100567 | 0215 | - | Wrong references from 36.521-1 clauses 8.4 and 8.5 into Annex G | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100569 | 0216 | - | Typos in 36.521-1, Annex E | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100571 | 0217 | - | Minimum test time for performance tests | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100572 | 0218 | - | Correction to 6.6.3.3 Additional spurious emissions | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100790 | 0219 | - | DL-RMC-s for transmitter tests: Corrections | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100800 | 0220 | - | Update of Test environment for RF test | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100803 | 0221 | - | Spectrum emission mask: Correction to uplink configuration | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100807 | 0222 | - | Performance tests: Scheduling of retransmissions | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100810 | 0223 | - | UL-RMC-s: Corrections and completion | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100814 | 0224 | - | Corrections to Cl 5.4.2.1 of TS 36.521-1 | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100815 | 0225 | - | LTE-RF: CR for UE configured UE transmitted output power test case | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100816 | 0226 | - | LTE-RF: CR for Power Control Relative power tolerance test case | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100822 | 0227 | - | CR to 36.521-1: Update to Maximum output power | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100823 | 0228 | - | CR to 36.521-1: Update to ACLR test case | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100825 | 0229 | - | CR to 36.521-1: Update to Additional Tx spurious emissions test case | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100826 | 0230 | - | RMC-s and OCNG patterns: Update according 36.101 8.8.0 | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100827 | 0231 | - | Receiver and performance tests: Update use of OCNG according 36.101 8.8.0 | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100828 | 0232 | - | Update of PDSCH Demodulation Tests | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100831 | 0233 | - | Introduction of clause 8.2.1.2, 8.2.1.3, 8.2.1.4 test case uncertainties and Test Tolerances | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100832 | 0234 | - | Clarifications on DRS performance test case | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100833 | 0235 | - | Misc update on MAC padding in PDCCH, CSI test | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100834 | 0236 | - | Updates to the TDD portion of CQI reporting test cases under AWGN | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100838 | 0237 | - | Editorial Correction to 8.2.1.3 | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100839 | 0238 | - | Update on Annex C for 36.521-1 | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100840 | 0239 | - | Update on MAC padding in TDD PMI test case 9.4 of 36.521-1. | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100841 | 0240 | - | Correction to CQI test cases under AWGN conditions | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100842 | 0241 | - | Correction to CQI test cases under fading conditions | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100843 | 0242 | - | Correction to PMI reporting test cases | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100845 | 0243 | - | CSI: Corrections to tests titles and RI clause structure | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100848 | 0244 | - | CR to 36.521-1: Update LTE RF test cases with test requirements for extended LTE1500MHz | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100886 | 0245 | - | Transmitter characteristics: UE Categories and other corrections | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100887 | 0246 | - | CR to 36.521-1: Update to Tx spurious emissions and Spurious emission band UE co-existence | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100888 | 0247 | - | Clarification on notes in Max Power | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100889 | 0248 | - | Maximum input level: Corrections w.r.t. UE categories | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100891 | 0249 | - | Correction to PDCCH demodulation test cases | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100892 | 0250 | - | Correction to PHICH demodulation test cases | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100907 | 0251 | - | Update of RI reporting test case | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | R5-100909 | 0252 | - | Correction to set UL power in Rx TCs | 8.4.0 | 8.5.0 |
| 2010-03 | RAN#47 | - | - | - | Moved to v9.0.0 with no change | 8.5.0 | 9.0.0 |
| 2010-06 | RAN#48 | R5-103102 | 0253 | - | CR to 36.521-1: Update of EARFCN for band 21 | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103103 | 0254 | - | CR to 36.521-1: Update of A-MPR test case with band 21 | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103104 | 0255 | - | CR to 36.521-1: Update of Additional Spurious test case with band 21 | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103106 | 0256 | - | CR to 36.521-1: Update to ACLR test case | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103108 | 0257 | - | CR to 36.521-1: Update of Reference sensitivity level test case | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103226 | 0258 | - | CR to 36.521-1: Update of UE RF requirements for LTE, Band 20 | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103263 | 0259 | - | LTE-RF: Updates of PDCCH demodulation test cases (FDD and TDD) | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103265 | 0260 | - | LTE-RF:CR for TDD ACK/NACK feedback mode in CQI BLER test cases | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103288 | 0261 | - | PDCCH Aggregation level for RF tests | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103291 | 0262 | - | Update and correction to UE maximum output power requirements | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103293 | 0263 | - | Editorial correction in In-band blocking test | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103296 | 0264 | - | Correction to additional spectrum emission mask test configuration | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103300 | 0265 | - | Corrections to Uplink RMC-s | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103450 | 0266 | - | LTE-RF: editorial CR for TC 7.6.2 and 7.7 | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103471 | 0267 | - | Minimum test time for performance tests | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103476 | 0268 | - | EVM with exclusion period (annex) | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103521 | 0269 | - | CR on 36.521-1 for updating the "Reporting of Channel State Information" | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103525 | 0270 | - | CR on 36.521-1 for corrections in UE RF requirements | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103598 | 0271 | - | Correction to notes in Max Power | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103602 | 0272 | - | Clarification of measurement conditions for Rx spurious emission | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103726 | 0273 | - | CR to 36.521-1: Update of Spurious emission band UE co-existence test case | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103727 | 0274 | - | LTE-RF: CR for Prach time mask test case | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103728 | 0275 | - | LTE-RF: CR for General ON/OFF time mask test case | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103729 | 0276 | - | LTE-RF: Update to spectrum flatness test case and relevant annexes | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103730 | 0277 | - | LTE-RF:CR for test case of In-band emissions | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103731 | 0278 | - | EVM with exclusion period (test) | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103732 | 0279 | - | CR to 36.521-1 on Correction to Demodulation Requirements for PDSCH | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103733 | 0280 | - | CR to 36.521-1: Update PDCCH DCI Formats for Open Loop and Closed Loop Spatial Multiplexing Test Cases | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103751 | 0281 | - | Misc update in CSI tests | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103752 | 0282 | - | Correction of the statistical part in PMI and RI tests | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103753 | 0283 | - | LTE-RF:CR to downlink RMCs for TX characteristics | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103754 | 0284 | - | LTE-RF: Update of annex C | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103756 | 0285 | - | Measuring throughput ratios (Annex G) | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103763 | 0286 | - | LTE-RF: CR for Minimum output power test case | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103764 | 0287 | - | Performance, CSI reporting and uncertainties for UEs with multiple Rx antennas | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103771 | 0288 | - | Introduction of clause 8.4.1 and 8.5.1 test case uncertainties and Test Tolerances | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103778 | 0291 | - | Uplink power for receiver tests | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103780 | 0292 | 1 | Addition of the exceptional message for In-band emissions | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103781 | 0289 | - | Correction to 6.5.2.1 EVM | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103782 | 0290 | - | Correction to CQI reporting | 9.0.0 | 9.1.0 |
| 2010-09 | RAN#49 | R5-104090 | 0294 | - | Corrections to Spectrum emission mask test regarding UE category | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104091 | 0295 | - | Missing note in Additional spurious emission test with NS_07 | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104095 | 0296 | - | PDCCH Aggregation level for CSI tests | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104096 | 0297 | - | Default initial and connection Uplink power for RF tests | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104212 | 0298 | - | Limits on Uplink power for Receiver tests | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104244 | 0299 | - | Correction to Demodulation Requirements | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104461 | 0300 | - | CR to 36.521-1: Editorial Corrections for Closed Loop Spatial Multiplexing Test Cases | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104478 | 0301 | - | Correction to Test requirements in 6.5.2.2 | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104482 | 0302 | - | Correction to 8.2.1.1 | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104520 | 0303 | - | 36521-1 General update of sections 00 to 08: missing Introduction references formatting | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104583 | 0304 | - | No necessity to apply - consecutive time slots for EVM | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104584 | 0305 | - | Correction to E.4.4 EVM equalizer spectrum flatness | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104630 | 0306 | - | Correction of table reference in In-band emissions test | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104808 | 0307 | - | CR to 36.521-1: Update to Additional Spectrum Emission Mask test case | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104809 | 0308 | - | CR to 36.521-1: Update to Spurious emission band UE co-existence test case | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104810 | 0309 | - | LTE-RF: CR for Max Output Power | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104811 | 0310 | - | LTE-RF: CR for Freq Error | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104812 | 0311 | - | Introduction of exclusion period for PUCCH-EVM test in clause 6.5.3 | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104813 | 0312 | - | Correction to Demodulation UE-Specific Reference Symbols | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104814 | 0313 | - | Uncertainties and Test Tolerances for CSI Test cases 9.2.1.1 and 9.2.1.2 | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104815 | 0314 | - | Uncertainties and Test Tolerances for CSI Test cases 9.2.2.1 and 9.2.2.2 | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104816 | 0315 | - | UE applicability for CSI test cases | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104817 | 0316 | - | Update of CQI reporting TCs under fading conditions | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104818 | 0317 | - | Update of Reporting of Precoding Matrix Indicator TCs | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104819 | 0318 | - | Correction of the statistical part 9.3.1.1.1 (CQI Reporting under fading conditions) | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104820 | 0319 | - | Correction of the statistical part 9.3.3.1.1 (CQI Reporting under fading conditions) | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104821 | 0320 | - | Correction of the statistical part 9.3.2.1.1 (CQI Reporting under fading conditions) | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104822 | 0321 | - | Update and new RMC-s for CQI tests | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104823 | 0322 | - | Correction of EVM calculation in annex | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104824 | 0323 | - | Introduction of exclusion period for PUCCH-EVM test in Annex E | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104844 | 0324 | - | Pcmax changes to Configured UE Transmitted Output Power | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104845 | 0325 | - | Clarification on the frequency range with net work signal in 6.6.3.2 | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104846 | 0326 | - | Update of editor's notes | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104847 | 0327 | - | Removal of Extreme Conditions in 6.2.3 | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104850 | 0328 | - | Corrections to Test procedure loop in CSI tests | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104851 | 0329 | - | Introduction of TDD CQI Reporting under fading conditions and frequency-selective interference test case | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104852 | 0330 | - | Introduction of TDD RI Reporting test case | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104853 | 0331 | - | Update of CQI reporting TCs under AWGN conditions | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104854 | 0332 | - | Update of FDD RI Reporting TC | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104857 | 0333 | - | CR to 36.521-1 LTE UE Tx_RX test cases band 20 | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104861 | 0334 | - | Corrections to Test requirements for MPR test | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104863 | 0335 | - | Clarification on notes in Max Power | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104872 | 0336 | - | Correction to 6.3.5.2 | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104873 | 0337 | - | Numbering and alignment of TDD PHICH demod test cases | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104874 | 0338 | - | Correction to test numbering for exceptional messages in 8.2.x.x | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104875 | 0339 | - | Correction to 9.2.2 | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104876 | 0340 | - | Correction to the test procedures of 9.3.x | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104877 | 0341 | - | Correction to 9.4.x | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104878 | 0342 | - | The new reference of connection diagram for 9.3.3 | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104879 | 0343 | - | Correction to 6.3.4.1 and 6.3.5.1 | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104888 | 0344 | - | Update of Annex C.2 for AG level | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-105055 | 0345 | - | Introduction of a new RF test case (8.7) to verify downlink sustained data rate performance | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-105061 | 0347 | - | CR to 36.521-1: Modification to Additional Maximum Power Reduction Test Case | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-105062 | 0348 | - | Modification to Additional Spectrum Emission Mask | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-105063 | 0349 | - | Modification to Additional Spurious Emissions | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-105064 | 0350 | - | Modification to Maximum Power Reduction | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-105065 | 0351 | - | Modification to Adjacent Channel Leakage Power Ratio | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | RP-100987 | 0352 | - | Correction of status for RF performance test case | 9.1.0 | 9.2.0 |
| 2010-12 | RAN#50 | R5-106073 | 0353 | - | Corrections to receiver spurious emissions test | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106074 | 0354 | - | Update of downlink power for receiver tests | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106076 | 0355 | - | CQI: Side condition when CQI median equals min or max CQI-values | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106077 | 0356 | - | Update of the throughput-definition for multi-data stream transmission | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106078 | 0357 | - | Update of RF OCNG patterns | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106092 | 0358 | - | Correction of DCI format used in PDSCH performance test 8.2.1.4.2 | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106249 | 0359 | - | CR to 36.521-1: Correction to Spurious emission band UE co-existence test case | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106250 | 0360 | - | CR to 36.521-1: Correction to Additional Tx spurious emissions test case | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106374 | 0361 | - | Correction of FDD CQI reporting test under AWGN - PUCCH 1-1 | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106394 | 0362 | - | Correction of clause 9.3.1 and 9.3.3 | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106399 | 0363 | - | "Correction of G.2.5 Pass fail decision rules" | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106420 | 0364 | - | Introduction of test uncertainties and tolerances for TDD PDSCH DRS test cases | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106440 | 0365 | - | Correction to unsigned numbers in Annex F.1.2 | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106443 | 0366 | - | Correction to the exceptional messages in 9.4 Reporting of PMI TCs | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106491 | 0367 | - | CR to 36.521-1: Correction to Table Numbering Error in TDD PDSCH Closed Loop Single/Multi Layer Spatial Multiplexing 2x2 | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106512 | 0368 | - | Transport format table clarification in CSI test cases | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106801 | 0369 | - | HARQ scheduling in TDD performance tests using special subframes | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106803 | 0370 | - | Correction to Fading Profiles in TCs 8.4 and 8.5 | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106816 | 0372 | - | CR to 36.521-1: Modification to Spectrum Emissions Mask | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106817 | 0373 | - | Introduction of test uncertainties and tolerances for TDD PCFICH/PDCCH and PHICH test cases | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106818 | 0374 | - | Update of TDD PDSCH CRS Demodulation test cases | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106821 | 0375 | - | PUSCH-EVM with exclusion period | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106822 | 0376 | - | Maintenance of Band 20 for receiver tests | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106823 | 0377 | - | Completion of clause 9.3.1 and 9.3.2 | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106824 | 0378 | - | Update of FDD RI Reporting TC | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106825 | 0379 | - | Correction to 9.2.2 CQI TCs | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106826 | 0380 | - | G.3.5 on PDCCH&PHICH Minimum Test Times | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106827 | 0381 | - | Completion of test time and ╬│-TT for clauses 9.3 to 9.5 | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106828 | 0382 | - | EVM window length for PRACH | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106842 | 0383 | - | Modification of TC 8.3.2.1 - TDD PDSCH Single-layer SM Performance (UE-Specific Reference Symbols) | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106843 | 0384 | - | Power control relative power tolerance: Missing band edge relaxation | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106844 | 0385 | - | SRS time mask test procedure update | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106845 | 0386 | - | Correction of TC General ON/OFF time mask | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106846 | 0387 | - | Update of TDD CQI reporting test under frequency selective interference conditions | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106847 | 0388 | - | Update of TDD RI reporting test | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106848 | 0389 | - | Iot setting in CQI test clarification | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106850 | 0390 | - | Correction of Test Uncertainties and Test Tolerances for Reference Sensitivity-Band 4 | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106855 | 0391 | - | Correction to DL and UL RMC configurations in 6.5.1 Frequency Error | 9.2.0 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106858 | 0392 | - | CR to 36.521-1: Update LTE RF test cases with test requirements for EUTRA TDD LTE band 41. | 9.2.0 | 9.3.0 |
| 2011-03 | RAN#51 | R5-110138 | 0393 | - | Spurious emission band co-existence test: Remaining old test requirement table | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110139 | 0394 | - | Transmit intermodulation test: Interferer offset for Band 20, Bandwidth 20 MHz | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110141 | 0395 | - | PMI Performance tests: Corrections to test settings | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110143 | 0396 | - | PDCCH Performance test: Correction to TDD DL RMC-s | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110144 | 0397 | - | OCNG for RF tests: Updates | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110158 | 0398 | - | CQI Performance tests: Clarification on subbands used in the test | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110302 | 0399 | - | LTE RF: PCFICH/PDCCH Transmit Diversity Performance | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110349 | 0401 | - | Correction of OFF power measurements in 6.3.4 ON/OFF time mask | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110354 | 0402 | - | Correction of Annex E (Global In-Channel TX-Test) | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110506 | 0403 | - | Addition of 3500 MHz TDD bands into chapter 5 of 36.521-1 | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110747 | 0404 | - | Updates to section 8.7 DL sustained data rate test case | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110850 | 0405 | - | Sustained data rate: Definition of UL RMC-s | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110860 | 0400 | - | Update measurement period from test procedure of 6.2.2 | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110861 | 0428 | - | A-MPR Test requirement update | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110862 | 0437 | - | Measuring throughput with different payload size. | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110900 | 0406 | - | Correction to SNR reference in FDD PCFICH/PDCCH Single Antenna Port Performance Test | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110901 | 0407 | - | CR to 36.521-1: Correction of Additional Maximum Power Reduction (A-MPR) test case | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110915 | 0408 | - | Uncertainties and Test Tolerances for CSI Test cases 9.3.1.1.x and 9.3.2.1.x | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110916 | 0409 | - | Uncertainties and Test Tolerances for CSI Test cases 9.3.3.1.x | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110917 | 0410 | - | Uncertainties and Test Tolerances for CSI Test cases 9.4.1.1.x and 9.4.2.1.x | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110918 | 0411 | - | Uncertainties and Test Tolerances for CSI Test cases 9.5.1.x | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110919 | 0412 | - | Editorial errors in CSI test cases 9.2.1.x Test procedure | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110933 | 0413 | - | Handling of different releases in RAN5 LTE RF specification | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110938 | 0414 | - | PRACH Time mask and EVM tests: TDD Special subframe configuration | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110939 | 0415 | - | Update on PRACH time mask for TDD | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110940 | 0416 | - | Correction of the connection diagram reference in the initial conditions for Transmitter Characteristics | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110941 | 0417 | - | Update of the reference sensitivity requirement for the 1.4MHz and 3MHz bandwidths and note correction for Band 3 and Band 9 | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110942 | 0418 | - | CR Removing brackets of band 41 reference sensitivity in 36.521-1 | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110949 | 0419 | - | PDCCH and PHICH performance tests: Updates and corrections | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110950 | 0420 | - | Addition of CodeBookSubsetRestriction bitmap for Multi-Layer Spatial Multiplexing | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110951 | 0421 | - | Addition of exceptional message in 8.4.1.2.2 | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110952 | 0422 | - | CQI test 9.2.2.2: Update acc TS 36.101 | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110953 | 0423 | - | Maximum input level test: Correction to DL-RMC | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110954 | 0424 | - | Correction of E.7, EVM with exclusion period | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110967 | 0425 | - | Updates to Additional Spurious emissions and Spurious emission band UE co-existence test | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110970 | 0426 | - | LTE RF: references to state 3A in 36.521-1 | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110973 | 0429 | - | RI Performance tests: Corrections | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110975 | 0430 | - | Completion of annex G.3.5 (Minimum test time, performance tests) | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110978 | 0431 | - | Correction to Band 12 frequency range | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110979 | 0432 | - | Additional in-band blocking requirement for Band 12 | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110989 | 0433 | - | Completion of annex G.3.6 (test conditions, performance tests) | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110990 | 0434 | - | Addition of test cases of TDD PDSCH Single-layer and Dual-layer Spatial Multiplexing Performance | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110991 | 0435 | - | Correction to Times Mask and Power Control tests | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110992 | 0436 | - | Add requirement of QPSK with partial RB allocation into test requirement of 6.2.3 | 9.3.0 | 9.4.0 |
| 2011-04 | - | - | - | - | Added approved R5-110967 which was missing. | 9.4.0 | 9.4.1 |
| 2011-06 | RAN#52 | R5-112148 | 0438 | - | ON/OFF time mask for PRACH: PRACH configuration index | 9.4.1 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112149 | 0439 | - | CQI tests with frequency selective scheduling mode: Random selection of Sub-Bands | 9.4.1 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112150 | 0440 | - | DL-RMC for receiver tests: Obsolete editors note | 9.4.1 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112211 | 0441 | - | Adding Band 24 to TS 36.521-1 | 9.4.1 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112311 | 0442 | - | Correction to the sustained data rate tests | 9.4.1 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112312 | 0443 | - | Correction to the multi-antenna transmission tests | 9.4.1 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112349 | 0444 | - | Update Annex G.3.6 for spatial multiplexing test cases | 9.4.1 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112412 | 0445 | - | Correction to DL RMC for Max input for UE category 2 | 9.4.1 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112459 | 0446 | - | EVM window length for PRACH | 9.4.1 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112460 | 0447 | - | Removal of square brackets in Annex G.5.4 | 9.4.1 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112462 | 0448 | - | Test tolerances for EVM with exclusion period | 9.4.1 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112808 | 0449 | - | LTE RF - UEs Demodulation and CSI Band applicability | 9.4.1 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112810 | 0450 | - | Correction on test requirement of test case 6.5.2.3 | 9.4.1 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112811 | 0451 | - | LTE RF - TC6.3.5.2 Change in Minimum Requirements | 9.4.1 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112812 | 0452 | - | Removable of NS values for TDD | 9.4.1 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112813 | 0453 | - | Correction on initial condition for test cases of 9.3.3 | 9.4.1 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112814 | 0454 | - | Correction to 9.4 PMI test cases | 9.4.1 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112816 | 0455 | - | Test procedure change on Tx spurious emissions test cases | 9.4.1 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112850 | 0456 | - | Update of Initial conditions according to table 7.3.3-2 for the 5MHz and 10MHz bandwidths | 9.4.1 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112851 | 0457 | - | Iot modelling in frequency-selective interference CQI tests | 9.4.1 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112852 | 0458 | - | Correction to test procedure in 9.5.1 | 9.4.1 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112859 | 0459 | - | Introduction of additional PDCCH test cases for Rel-9 | 9.4.1 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112860 | 0460 | - | Introduction of additional PHICH test cases for Rel-9 | 9.4.1 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112861 | 0461 | - | Introducing UE-selected subband CQI tests for Rel-9 | 9.4.1 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112862 | 0462 | - | Introduction of new PMI test cases for UE-selected sub-band reporting for Rel-9 | 9.4.1 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112863 | 0463 | - | Addition of a new TC for FDD MBMS performance | 9.4.1 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112864 | 0464 | - | Addition of a new TC for TDD MBMS performance | 9.4.1 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112866 | 0465 | - | Addition of UE category 1 coverage for single-layer spatial multiplexing transmission on antenna port 7 or 8 | 9.4.1 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112870 | 0466 | - | Removal of square brackets in annex G.3.5 (Minimum test time, performance tests) | 9.4.1 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112871 | 0467 | - | Update minimum requirements and test requirements for spatial multiplexing test cases | 9.4.1 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112873 | 0468 | - | Test procedure change on few Rx test cases | 9.4.1 | 9.5.0 |
| 2011-09 | RAN#53 | R5-113178 | 0506 | - | RF TC 8.7.2: Correction of the ACK/NACK feedback mode | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-113179 | 0469 | - | RF TC-s 9.3.4: Updates to minimum requirements and corrections to test procedures | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-113180 | 0470 | - | RF: New UL RMC for 10 MHz 15RB-s | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-113181 | 0471 | - | RF: Use of State 3A-RF | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-113233 | 0472 | - | LTE RF: EVM Annex E correction | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-113238 | 0473 | - | RF TC 6.3.4.2.2: Correction to SRS configuration | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-113368 | 0474 | - | Correction in test cases of 8.2.2.1.2 and 8.5.1 | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-113369 | 0475 | - | Correction in test cases of 9.5 | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-113380 | 0476 | - | Update of minimum test time for PCFICH/PDCCH and PHICH tests | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-113383 | 0477 | - | Correction of Table G.3.5-1 (minimum test time) | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-113419 | 0478 | - | Correction for spurious emission band UE co-existence limits of Band 3 | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-113454 | 0479 | - | Correction to test procedure in 6.5.1 | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-113455 | 0480 | - | Correction to Minimum requirement in 6.6.2.2 | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-113456 | 0481 | - | Correction to RMC for PDCCH/PCFICH performance requirements | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-113458 | 0482 | - | Correction to CSI TCs | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-113541 | 0483 | - | Introduction of Expanded 1900MHz Band (Band 25) into section 5 of 36.521-1 | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-113596 | 0506 | - | Abbreviation update and Editorial corrections in TS 36.521-1 | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114000 | 0484 | - | Correction in 6.3.5.2 Power Control Relative power tolerance | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114001 | 0485 | - | Correction in 6.3.4.2.2 SRS time mask | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114002 | 0486 | - | Addition of PDSCH TDD performance tests for Low UE categories | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114003 | 0487 | - | Additional FDD scenarios | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114004 | 0488 | - | Addition of RMC-s for PDSCH performance tests for low UE categories | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114032 | 0489 | - | Introduction of Expanded 1900MHz Band (Band 25) into section 6 of 36.521-1 | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114033 | 0490 | - | Correction on TDD MBMS performance requirements for 64QAM mode | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114034 | 0491 | - | Correction on FDD MDMS performance requirements for 64QAM mode | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114038 | 0492 | - | RF TC 6.3.5.2: Consideration of band edge relaxation in test requirements | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114039 | 0493 | - | Band 19 A-MPR refinement | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114040 | 0494 | - | Test system uncertainty and TT for 6.6.3.3 additional spurious emissions NS_07 | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114041 | 0495 | - | Correction to Additional Maximum Power Reduction | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114042 | 0496 | - | Correction to 6.3.4.2.1 PRACH time mask | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114043 | 0497 | - | Correction to Receiver Characteristics Minimum Requirements | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114044 | 0498 | - | Addition of PDSCH FDD performance tests for Low UE categories | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114045 | 0499 | - | Additional Rel-9 TDD scenarios | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114046 | 0500 | - | Update of CQI/PMI test cases | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114047 | 0501 | - | Update of RI test cases | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114048 | 0502 | - | Release dependent RMCs in PCFICH/PDCCH and PHICH tests | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114049 | 0503 | - | Addition of tests scenarios in Annex | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114073 | 0504 | - | Introduction of Expanded 1900MHz Band (Band 25) into section 7 of 36.521-1 | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114091 | 0505 | - | Correction to test cases 10.1 and 10.2 | 9.5.0 | 9.6.0 |
| 2011-12 | RAN#54 | R5-115112 | 0507 | - | RF: Coverage of MPR and A-MPR requirements in emissions tests | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115113 | 0508 | - | RF: Update to names of some RMC-s used in different releases | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115114 | 0509 | - | RF TC 6.2.4, 6.6.2.2, 6.6.3.3: Corrections to A-MPR related tests | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115115 | 0510 | - | RF TC-s 9: Corrections to UL allocation for some CQI tests | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115116 | 0511 | - | RF TC 6.3.5.2: Missing extreme conditions in test requirements | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115117 | 0512 | - | RF TC 6.2.3: Missing test requirements for band 25 | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115118 | 0513 | - | RF TC 6.5.2.1: Redundant conflicting IE-s in message contents | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115119 | 0514 | - | RF TC 6.6.3.1: Correction of reference to the connection diagram | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115324 | 0517 | - | LTE RF: CSI test case update | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115472 | 0518 | - | Corrections to the dual-layer beamforming demodulation requirements | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115475 | 0519 | - | Correction to Code Block Numbers | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115800 | 0522 | - | Delete note in PUSCH-EVM | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115812 | 0526 | - | RF: Corrections to tests with release dependent requirements | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115815 | 0527 | - | Test Frequency for Relative Power Tolerance | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115821 | 0531 | - | RF: General review of the reference measurement channels | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115822 | 0532 | - | Correction for codebook subset restriction in single-layer closed loop spatial multiplexing test | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115830 | 0533 | - | Uncertainties and Test Tolerances for 3000MHz to 4200MHz, Tx Test cases | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115831 | 0534 | - | Uncertainties and Test Tolerances for 3000MHz to 4200MHz, Rx Test cases | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115832 | 0535 | - | Support for band 22, 42 and 43 | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115837 | 0536 | - | Update LTE RF test cases with test requirements for FDD LTE Band 23 in 36.521-1 | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115875 | 0537 | - | Correction on FDD and TDD MBMS conformance requirements | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115877 | 0538 | - | Introduction of test system uncertainties and TT to new TCs in clauses 9.3.4 to 9.4.2.2.2 | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115879 | 0539 | - | Uncertainties and Test Tolerances for Sustained data rate test cases | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115891 | 0540 | - | Requirement change in UE spurious emissions for Band 7 and 38 co-existence (Rel-8 only) | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115162 | 0515 | - | Definitions, symbols and abbreviations for CA RF | 9.7.0 | 10.0.0 |
| 2011-12 | RAN#54 | R5-115565 | 0520 | - | In-band blocking for CA (New) | 9.7.0 | 10.0.0 |
| 2011-12 | RAN#54 | R5-115568 | 0521 | - | Spurious Response for CA (New) | 9.7.0 | 10.0.0 |
| 2011-12 | RAN#54 | R5-115801 | 0523 | - | Out-of-Band blocking for CA (new) | 9.7.0 | 10.0.0 |
| 2011-12 | RAN#54 | R5-115802 | 0524 | - | Narrow band blocking for CA (new) | 9.7.0 | 10.0.0 |
| 2011-12 | RAN#54 | R5-115803 | 0525 | - | Addition of new RMCs for CA feature related Chapter8 test cases | 9.7.0 | 10.0.0 |
| 2011-12 | RAN#54 | R5-115816 | 0528 | - | General Section for CA RF | 9.7.0 | 10.0.0 |
| 2011-12 | RAN#54 | R5-115817 | 0529 | - | Frequency Band and Channel Assignments for CA RF | 9.7.0 | 10.0.0 |
| 2011-12 | RAN#54 | R5-115818 | 0530 | - | UE Maximum Output Power for intra-band contiguous CA (new) | 9.7.0 | 10.0.0 |
| 2012-03 | RAN#55 | R5-120134 | 0541 | - | RF: General clarification on the count of ACK / NACKs in throughput calculation in CQI tests | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120137 | 0542 | - | RF: Corrections to message contents in some MIMO demodulation performance tests | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120138 | 0543 | - | RF: Updates to handling requirements and tests for different releases | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120140 | 0544 | - | RF: Coverage of MPR and A-MPR requirements in Tx emissions tests | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120243 | 0545 | - | Tx Test cases Uplink power limit window for 3000MHz to 4200MHz | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120244 | 0546 | - | Rx Test cases Uplink power limit window for 3000MHz to 4200MHz | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120252 | 0547 | - | Global in-channel TX-test, delete note | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120288 | 0548 | - | LTE RF - Update of abbreviations and addition of RBstart | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120314 | 0549 | - | Clarification for the starting point of RB in 6.3.5.2 Relative Power test | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120338 | 0550 | - | Correction to MCS value in Table A.4-3 | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120343 | 0551 | - | Introduction of new maximum input level test case for CA | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120351 | 0552 | - | CA RF - Changes in aggregated CC configurations | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120355 | 0553 | - | CA RF - updates to 7.7A Spurious response for CA | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120370 | 0554 | - | Test Tolerance for 6.6.3.3 additional spurious emissions NS_07 | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120426 | 0555 | - | Correction to Frequency Range for Spurious Emission Requirements | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120521 | 0556 | - | Introduction to Maximum Power Reduction for CA | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120524 | 0557 | - | Introduction to Maximum Power Reduction for CA in Annex | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120526 | 0558 | - | Introduction to Adjacent Channel Leakage power Ratio (ACLR) for CA in Annex | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120801 | 0559 | - | Correction of TC 6.5.2.1 Error Vector Magnitude (EVM) for Test requirement | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120802 | 0560 | - | Correction of TC 8.2.1.3.1 FDD PDSCH Open Loop Spatial Multiplexing 2x2 for 8.2.1.3.1.4.1 Initial conditions | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120803 | 0561 | - | Correction of TC8.2.1.3.2 FDD PDSCH Open Loop Spatial Multiplexing 4x2 for 8.2.1.3.2.4.1 Initial conditions | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120804 | 0562 | - | Correction of TC 8.2.2.3.1 TDD PDSCH Open Loop Spatial Multiplexing 2x2 for 8.2.2.3.1.4.1 Initial conditions | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120805 | 0563 | - | Correction of TC 8.2.2.3.2 TDD PDSCH Open Loop Spatial Multiplexing 4x2 for 8.2.2.3.2.4.1 Initial conditions | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120816 | 0564 | - | Addition of frequency band and channel assignments for UL MIMO | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120825 | 0565 | - | Introduction of Chapter8 Single antenna port and Open loop spatial Multiplexing test cases for CA capable UE's. | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120828 | 0566 | - | LTE RF - UE Co-ex test point clarification for bands 7 and 38 | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120833 | 0567 | - | Addition of receiver image section 7.10 | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120835 | 0568 | - | Correction to Reporting of Rank Indicator (RI) FDD Test | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120837 | 0569 | - | Introduction to Adjacent Channel Leakage power Ratio (ACLR) for CA | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120839 | 0570 | - | Addition of a new TC 6.2.2B for UE Maximum Output Power for UL-MIMO | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120842 | 0571 | - | Harmonic exceptions in LTE UE to UE co-existence tests | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120843 | 0572 | - | Clarification for upper and lower interferers in 7.6.3 Narrow Band Blocking | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120844 | 0573 | - | RF: Correction of frequency range for out of band blocking test | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120845 | 0574 | - | Correction and completion to clause 10 MBMS performance | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120874 | 0575 | - | RF: New RMC-s and updates to the RMC-s overview tables | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120875 | 0576 | - | RF: Test frequencies for UE co-existence emissions under Note 13 | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120877 | 0577 | - | UE Transmit OFF power for CA | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120878 | 0578 | - | Frequency error for Intraband CA | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120879 | 0579 | - | Occupied bandwidth for intra-band contiguous CA | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120880 | 0580 | - | Transmitter Spurious emissions for CA | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120881 | 0581 | - | Reference sensitivity level for CA | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120882 | 0582 | - | CA RF - Addition of test description to 6.2.2A MOP for intra-band CA | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120884 | 0583 | - | CA RF - updates to 7.6.2A OOB blocking for CA | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120885 | 0584 | - | CA RF - updates to 7.6.3A Narrow-band blocking for CA | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120899 | 0585 | - | CA RF - updates to 7.6.1A In-band Blocking for CA | 10.0.0 | 10.1.0 |
| 2012-03 | RAN#55 | R5-120900 | 0586 | - | Correction to UE Spurious Emissions | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-121138 | 0588 | - | Correction of TC 6.3.5.2 Power Control Relative power tolerance | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121155 | 0589 | - | Correction in test configuration table of sections 7.4 and 7.5 | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121206 | 0590 | - | Correction of test procedure for intermodulation | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121208 | 0591 | - | Correction of test procedure for out-of-band blocking | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121221 | 0592 | - | Relative frequency error for CA performance tests | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121241 | 0593 | - | RF: Corrections to derivation of test requirements for some test points of TC 6.2.4 | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121242 | 0594 | - | RF: Updates to Annex I (Handling of different releases) | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121253 | 0595 | - | RF: Clarification of test frequencies for UE co-existence emissions under Note 13 | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121254 | 0596 | - | RF: Minor correction to RMC reference in minimum test time annex for TC 8.3.2.1.3 | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121305 | 0597 | - | Removal of test description from Transmit off power for CA and introducing ON/OFF time mask for CA | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121322 | 0598 | - | Update of 6.3.4.2.2 | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121337 | 0599 | - | Clarifications to Maximum Power Reduction for CA | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121438 | 0600 | - | Modification to Configured UE transmitted Output Power | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121451 | 0601 | - | Correction of the Band 23 for UE coexistence requirements | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121459 | 0602 | - | Correction of RF Test case: 8.7.2.1 TDD sustained data rate performance | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121463 | 0603 | - | Introduction of Adjacent Channel Selectivity (ACS) for CA | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121475 | 0604 | - | LTE CA general updates to sections 3 - 5 | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121479 | 0605 | - | CA RF - TC 6.2.2A MOP updates | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121482 | 0606 | - | CA RF - 7.6.1A In-band blocking updates | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121483 | 0607 | - | CA RF - 7.6.2A Out-of-band blocking updates | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121484 | 0608 | - | CA RF - 7.6.3A Narrow-band blocking updates | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121485 | 0609 | - | CA RF - 7.7A Spurious response updates | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121488 | 0610 | - | LTE RF - TC 6.6.3.2 UE co-existence requirements correction | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121523 | 0611 | - | Correction to downlink RB allocation in 7.5.4 | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121525 | 0612 | - | Correction to test requirement in 9.2.2.2 | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121678 | 0613 | - | TS 36.521-1: Spurious emission band UE co-existence alignment | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121680 | 0614 | - | TS 36.521-1: Applicability correction for 9.3.2.1.1 and 9.3.2.1.2 | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121700 | 0615 | - | Suffix definition of Clause 5 for CA, UL-MIMO, and DL-MIMO for 36.521-1 R-10 | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121903 | 0616 | - | Update of Band 41 RF requirements to align with recent core specification changes | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121906 | 0617 | - | TS 36.521-1: Introduction of Band 26/XXVI in section 5 | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121907 | 0618 | - | TS 36.521-1: Band 26 Test points for A-MPR test | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121909 | 0619 | - | CA RF - Adding contents to TC 6.2.4A A-MPR for CA | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121918 | 0620 | - | TS 36.521-1: Introduction of Band 26/XXVI in section 6 | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121920 | 0621 | - | Corrections of test parameters and test procedure for ACS | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121921 | 0622 | - | TS 36.521-1: Introduction of Band 26/XXVI in section 7 | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121925 | 0623 | - | CA RF - addition of test case 8.7.2.1A TDD sustained data rate performance for CA | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121937 | 0624 | - | A-MPR: Band 20, NS_10 not tested | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121938 | 0625 | - | RF: Several corrections to sustained data rate TC-s 8.7.1 and 8.7.2 | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121939 | 0626 | - | RF: Corrections to CSI tests | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121944 | 0627 | - | Addition of a new TC 6.3.3B | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121945 | 0628 | - | Addition of a new TC 6.3.2B for Minimum Output Power for UL-MIMO | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121946 | 0629 | - | Addition of a new TC 6.3.4B | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121947 | 0630 | - | Addition of a new TC 6.5.1B for Frequency Error for UL-MIMO | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121948 | 0631 | - | Addition of a new TC 6.6.1B | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121949 | 0632 | - | Addition of a new TC 6.7B | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121950 | 0633 | - | Addition of a new TC for Configured transmitted power for UL-MIMO | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121951 | 0634 | - | Addition of a new TC for Power Control Absolute Power Tolerance for UL-MIMO | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121955 | 0635 | - | SEM for CA | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121956 | 0636 | - | Addition of a new TC for Carrier leakage for UL-MIMO | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121957 | 0637 | - | Addition of a new TC for In-band emissions for non allocated RB for UL-MIMO | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121958 | 0638 | - | Addition of a new TC for EVM equalizer spectrum flatness for UL-MIMO | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121959 | 0639 | - | Addition of a new TC for Reference sensitivity level for UL-MIMO | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121960 | 0640 | - | Addition of a new TC for Maximum input level for UL-MIMO | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121961 | 0641 | - | Addition of a new TC for Adjacent Channel Selectivity (ACS) for UL-MIMO | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121962 | 0642 | - | Addition of a new TC for In-band blocking for UL-MIMO | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121963 | 0643 | - | Addition of a new TC for Narrow band blocking for UL-MIMO | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121964 | 0644 | - | Addition of a new TC for Wide band Intermodulation for UL-MIMO | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121983 | 0645 | - | Introduction of Chapter8 Single antenna port and Open loop spatial Multiplexing TDD test cases for CA capable UEs | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121984 | 0646 | - | EVM and Global In-Channel TX-Test for intra band CA | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121985 | 0647 | - | RF: Corrections to applicability of some CA tests | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121986 | 0648 | - | Addition of test description to Occupied bandwidth for CA | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121987 | 0649 | - | Addition of test description to ACLR for CA | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121988 | 0650 | - | Addition of test description to Transmitter Spurious emissions for CA | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121989 | 0651 | - | Addition of test description to Reference sensitivity level for CA | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121994 | 0652 | - | Addition of Codebook Subset Restriction in 8.3.2.x | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121995 | 0653 | - | Update of 6.3.5.2 | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121996 | 0654 | - | TS 36.521-1: 9.4.1.2.1 and 9.4.1.2.2.1 test procedure correction | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-121997 | 0655 | - | TS 36.521-1: Minimum conformance requirements alignments for section 9 | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-122003 | 0656 | - | Addition of a new TC for Out-of-band blocking for UL-MIMO | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-122004 | 0657 | - | Addition of a new TC for Spurious response for UL-MIMO | 10.1.0 | 10.2.0 |
| 2012-06 | RAN#56 | R5-122006 | 0658 | - | Addition of a new TC for Error Vector Magnitude for UL-MIMO | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123136 | 0659 | - | Correction of TC 6.3.5.2 | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123146 | 0660 | - | Corrections to Transmitter Characteristics Requirements | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123148 | 0661 | - | Correction to SNR definition | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123150 | 0662 | - | RF: Several editorial corrections | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123161 | 0663 | - | RF: PDCCH Padding in Rx spurious emissions test | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123179 | 0664 | - | Introduction of eDL-MIMO to Propagation Conditions | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123209 | 0665 | - | RF: Alignment of RMC references in test procedure of CSI tests | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123234 | 0666 | - | LTE RF - Correction to 7.4 and 7.5 UL allocations | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123239 | 0667 | - | Addition of test description to CA Maximum input level test case | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123267 | 0668 | - | Correction to Occupied bandwidth for CA | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123268 | 0669 | - | Correction to Reference sensitivity level for CA | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123324 | 0670 | - | New Annex for: Statistical testing of receiver performance with throughput for CA | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123330 | 0671 | - | Correction to test parameters in 6.6.3.2 | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123332 | 0672 | - | Update of section 8 uncertainties and Test Tolerances for Rel-9 Tests | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123333 | 0673 | - | Clarification of RB allocation for DRS demodulation tests | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123334 | 0674 | - | Correction to uplink reference measurement channel | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123386 | 0675 | - | Addition of a new TC for power control relative power tolerance for UL-MIMO | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123387 | 0676 | - | Addition of a new TC for aggregate power control tolerance for UL-MIMO | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123392 | 0677 | - | Update carrier frequency and EARFCN | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123394 | 0678 | - | Update test case of UE maximum output power for UL-MIMO | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123395 | 0679 | - | Update test case of configured UE transmitted output power for UL-MIMO | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123397 | 0680 | - | Update test case of power control absolute power tolerance for UL- MIMO | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123507 | 0681 | - | Editorial corrections to blocking characteristics for CA | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123510 | 0682 | - | ACLR for UL-MIMO | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123521 | 0683 | - | TS 36.521-1: A-MPR and additional spurious corrections | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123522 | 0684 | - | TS 36.521-1: UL allocation clarification in PUSCH-EVM with exclusion period test | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123523 | 0685 | - | TS 36.521-1: Section 9 applicability revision (UE categories) | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123534 | 0687 | - | TS 36.521-1:A-MPR Test tolerances update for NS_12, NS_13, NS_14, NS_15 | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123547 | 0688 | - | Correction to the content of reference table number for TC 7.8.1 | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123783 | 0689 | - | Corrections to EVM and global in channel test for Intra-Band CA | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123850 | 0690 | - | Correction to ONOFF time mask for CA | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123851 | 0691 | - | Correction to Transmitter Spurious emissions for CA | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123852 | 0692 | - | New Annex for: Statistical testing of receiver characteristics with CA | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123853 | 0693 | - | Introduction of eDL-MIMO to measurement of performance requirements | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123854 | 0694 | - | Addition of a new TC for additional maximum power reduction (A-MPR) for UL-MIMO | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123855 | 0695 | - | Additional texts for Note2 in Spurious emission test | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123857 | 0696 | - | Additions to test requirements for Maximum Power Reduction for CA | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123859 | 0697 | - | Addition of a new TC for time alignment between transmitter branches for UL-MIMO | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123900 | 0698 | - | Changes associated with PHS band operation change for LTE | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123901 | 0699 | - | TS 36.521-1: PDSCH RMC for Rel-9 PCFICH test cases | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123902 | 0700 | - | TS 36.521-1:Adding missing UL RMCs for test 9.4.1.2.2 | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123910 | 0701 | - | MPR for UL-MIMO | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123911 | 0702 | - | SEM for UL-MIMO | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123912 | 0703 | - | ADDITIONAL SEM for UL-MIMO | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123915 | 0704 | - | RF: Update of Annex I | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123917 | 0705 | - | Correction of test procedure for 36.521-1 chapter 8 CA test cases | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123919 | 0706 | - | TS 36.521-1: New test cases for CQI reporting under fading conditions - PUCCH 1-0 | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123932 | 0707 | - | RF: Updates to transmission mode 8 test cases | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123933 | 0708 | - | RF: Several corrections to MBMS performance tests | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123934 | 0709 | - | Addition of RF Test case: 8.7.2.1_1 TDD sustained data rate performance (Rel-10 and forward) | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123943 | 0710 | - | Introduction of FDD PMI Reporting - PUSCH 1-2 (Multiple PMI) for eDL-MIMO | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123944 | 0711 | - | Introduction of TDD PMI Reporting - PUSCH 1-2 (Multiple PMI) for eDL-MIMO | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123945 | 0712 | - | FDD RI Reporting for eDL-MIMO | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123946 | 0713 | - | TDD RI Reporting for eDL-MIMO | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123947 | 0714 | - | Introduction of eDL-MIMO to CSI Reference Measurement Channels | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123948 | 0715 | - | Introduction of eDL-MIMO to structure of subclauses | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123950 | 0716 | - | RF-CA: Introduction of new specification structure for Tx tests for CA | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123951 | 0717 | - | CA RF - TC 6.2.2A update | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123952 | 0718 | - | CA RF - Updates to Blocking Test Cases | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123953 | 0719 | - | CA RF - TC 7.7A Spurious Response update | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123954 | 0720 | - | New RF TC for 36.521-1 _ 6.3.5A.1 Power Control Absolute power tolerance for CA | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123955 | 0721 | - | New RF TC for 36.521-1 _ 6.3.5A.2 Power Control Relative power tolerance for CA | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123956 | 0722 | - | Correction to Transmit OFF power for CA | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123957 | 0723 | - | Adjusting test procedure and test requirements to new Annex G.3A | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123958 | 0724 | - | Update of Test case 8.7.2.1A TDD sustained data rate performance for CA | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123961 | 0725 | - | Corrections to A-MPR for CA | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123962 | 0726 | - | Additions to test description for Adjacent Channel Selectivity (ACS) for CA | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123963 | 0727 | - | Introduction to UE configured transmitted output power for CA | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123964 | 0728 | - | Introduction of ACS and UE configured Tx output power for CA in Annex | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123969 | 0729 | - | Introduction Single-Layer Spatial Multiplexing for eDL-MIMO | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123970 | 0730 | - | updates to 6.6.2.1A SEM for CA | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123977 | 0731 | - | Addition of a new TC6.6.3B.1 for Transmitter Spurious emissions for UL-MIMO | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123978 | 0732 | - | Addition of a new TC6.6.3B.2 for Spurious emission band UE co-existence for UL-MIMO | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123979 | 0733 | - | Addition of a new TC6.6.3B.3 for Additional spurious emissions for UL-MIMO | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123980 | 0734 | - | Addition of a new TC for narrow band blocking for UL-MIMO | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123981 | 0735 | - | Update test case of general ON OFF time mask for UL-MIMO | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123982 | 0736 | - | Update test case of frequency error for UL-MIMO | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123983 | 0737 | - | Update test case of Error Vector Magnitude (EVM) for UL- MIMO | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123984 | 0738 | - | Update test case of carrier leakage for UL-MIMO | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123985 | 0739 | - | Update test case of in-band emissions for non allocated RB for UL-MIMO | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123986 | 0740 | - | Update test case of EVM equalizer spectrum flatness for UL-MIMO | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123987 | 0741 | - | Update test case of reference sensitivity level for UL-MIMO | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123988 | 0742 | - | Update test case of maximum input level for UL-MIMO | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123989 | 0743 | - | Update test case of in-band blocking for UL-MIMO | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123990 | 0744 | - | Update test case of out-of-band blocking for UL-MIMO | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123991 | 0745 | - | Update test case of spurious response for UL-MIMO | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123992 | 0746 | - | Update test case of wide band Intermodulation for UL-MIMO | 10.2.0 | 10.3.0 |
| 2012-09 | RAN#57 | R5-123994 | 0747 | - | Update test case of Adjacent Channel Selectivity (ACS) for UL-MIMO | 10.2.0 | 10.3.0 |
| 2012-12 | RAN#58 | R5-125109 | 0749 | - | Update of TC6.6.3B.2 for Spurious emissions band UE co-existence for UL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125110 | 0750 | - | Update of TC6.6.3B.3 for Additional spurious emissions for UL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125146 | 0751 | - | Corrections to Test Procedure of TC 6.6.2.3 ACLR | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125147 | 0752 | - | Updates for TC 6.2.3B MPR for UL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125148 | 0753 | - | Updates for TC 6.6.2.1B SEM for UL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125149 | 0754 | - | Updates for TC 6.6.2.2B A-SEM for UL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125150 | 0755 | - | Updates for TC 6.6.2.3B ACLR for UL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125215 | 0756 | - | Correction of TC 6.6.2.1 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125252 | 0757 | - | Adding normative reference for core requirements in TC 6.2.4 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125253 | 0758 | - | Adding normative reference for core requirements for many section 9 tests | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125254 | 0759 | - | Correction of table references in sub clause G.X | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125260 | 0760 | - | CR to TC 7.4A: Splitting the CA Maximum input level test case | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125263 | 0761 | - | RF Cl G.3A: General corrections and completion of Annex G.3A with TDD tests | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125304 | 0762 | - | Updates for Minimum requirements of 6.2.5 Configured UE transmitted Output Power | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125309 | 0763 | - | Addition of Band 28 definition to Chap.5 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125311 | 0764 | - | Addition of Band 28 to 6.2.2 MOP | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125314 | 0765 | - | Addition of Band 28 to 7.3 Refsens | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125321 | 0766 | - | Updates for TC6.6.1A: Occupied bandwidth for CA | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125322 | 0767 | - | Updates for TC6.6.2.3A: ACLR for CA | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125323 | 0768 | - | Updates for TC6.6.3.1A: Transmitter Spurious emissions for CA | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125346 | 0769 | - | TC 6.2.4A.1 A-MPR for CA updates | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125351 | 0770 | - | Correction to LTE Rx TCs UL allocation location | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125365 | 0771 | - | Uncertainties and Test Tolerances for Rel-9 CQI Reporting under fading conditions, PUCCH 1-0 Tests | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125374 | 0772 | - | Clean up of Tx tests | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125405 | 0773 | - | Addition of 15MHz and 20MHz Bandwidths for Band 23 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125407 | 0774 | - | RF TC 6.6.3.2: Clarification and corrections | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125417 | 0775 | - | Additions to test description for TC 7.5A.1 Adjacent Channel Selectivity (ACS) for CA (intra-band contiguous DL CA and UL CA) | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125422 | 0776 | - | Introduction to CA_1 & CA_40 TC 7.5A.2 Adjacent Channel Selectivity (ACS) for CA (intra-band contiguous DL CA without UL CA) | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125443 | 0777 | - | Corrections of RMC for receiver characteristics | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125445 | 0778 | - | Corrections of FRC subframe allocations for performance requirements | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125450 | 0779 | - | Introduction of TC 9.2.2.1_D FDD CQI Reporting under AWGN conditions - PUCCH 1-1 for eDL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125451 | 0780 | - | Introduction of TC 9.2.2.2_D TDD CQI Reporting under AWGN conditions - PUCCH 1-1 for eDL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125452 | 0781 | - | Introduction of TC 9.3.2.2.1_D FDD CQI Reporting under fading conditions - PUCCH 1-1 for eDL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125453 | 0782 | - | Introduction of TC 9.3.2.2.2_D TDD CQI Reporting under fading conditions - PUCCH 1-1 for eDL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125454 | 0783 | - | Additions of eDL-MIMO to Annex A | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125455 | 0784 | - | Additions of eDL-MIMO to Annex B Propagation Conditions for eDL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125456 | 0785 | - | Additions of eDL-MIMO to Annex F | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125463 | 0786 | - | Additions to TC 9.4.2.1.1_D FDD PMI Reporting - PUSCH 1-2 (Multiple PMI) for eDL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125464 | 0787 | - | Additions to TC 9.4.2.1.2_D TDD PMI Reporting - PUSCH 1-2 (Multiple PMI) for eDL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125465 | 0788 | - | Additions to TC 9.5.1.1_D FDD RI Reporting for eDL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125466 | 0789 | - | Additions to TC 9.5.1.2_D TDD RI Reporting for eDL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125593 | 0790 | - | RF TC 6.2.4: Minor correction to test points for NS_15 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125594 | 0791 | - | RF TC 6.2.4B: Minor correction to test points for NS_15 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125610 | 0792 | - | TS 36-521-1: MPR correction for band 26 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125801 | 0793 | - | Updates of 6.3.3A UE Transmit OFF power for CA | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125802 | 0794 | - | Update of Annex F in 36.521-1 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125803 | 0795 | - | Updates for 6.3.4A ON/OFF time mask for CA | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125804 | 0796 | - | Addition of Transmitter test case - Transmit intermodulation for CA | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125835 | 0797 | - | Update of TC6.6.3B.1 for Transmitter Spurious emissions for UL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125851 | 0798 | - | Introduction of TC 8.3.1.2.1_D FDD PDSCH Dual-layer Spatial Multiplexing for eDL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125852 | 0799 | - | Introduction of TC 8.3.2.2.1_D TDD PDSCH Dual-layer Spatial Multiplexing for eDL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125853 | 0800 | - | Additions to Annex G for eDL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125854 | 0801 | - | Corrections to Annex H for eDL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125855 | 0802 | - | Additions to TC 8.3.2.1.2_D TDD Single-Layer Spatial Multiplexing on antenna ports without a simultaneous transmission for eDL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125856 | 0803 | - | Additions to TC 8.3.2.1.3_D TDD Single-Layer Spatial Multiplexing on antenna ports with a simultaneous transmission for eDL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125857 | 0804 | - | Introduction of Band 27 to TS 36.521-1 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125858 | 0805 | - | Updates to TC 8.2.2.3.1_A.1 for CA | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125859 | 0806 | - | Addition of new test case 8.2.1.4.2_A.1 for CA | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125860 | 0807 | - | Addition of new test case 8.2.2.4.2_A.1 for CA | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125861 | 0808 | - | Addition of sustained downlink data rate TC 8.7.1.1_A.1 for CA | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125862 | 0809 | - | Addition of sustained downlink data rate TC 8.7.1.1_A.2 for CA | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125866 | 0810 | - | Update of RF TC for 36.521-1 _ 6.3.5A.2.1 Power Control Relative power tolerance for CA | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125867 | 0811 | - | Update TC 6.6.3.2_1 Spurious emission band UE co-existence(Release 9 and forward) | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125868 | 0812 | - | Correction to test applicability in 6.2.4 A-MPR test case | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125869 | 0813 | - | Correction to test parameter in Perf 8.2.1 of 36.521-1 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125870 | 0814 | - | Clarification of AG level for 8.7 sustained data rate test of 36.521-1 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125871 | 0815 | - | Correction to SNR test points in test procedure | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125872 | 0816 | - | Clarification of random precoding granularity | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125873 | 0817 | - | Update of General sections for CA | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125874 | 0818 | - | Introduction to CA_1-5 TC 7.5A.3 Adjacent Channel Selectivity (ACS) for CA (inter-band DL CA without UL CA) | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125875 | 0819 | - | Addition of a new TC 6.3.5A.3.1 Aggregate power control tolerance for CA(intra-band contiguous DL CA and UL CA) | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125876 | 0820 | - | Addition of a new TC 6.6.3.2A.1 Spurious emission band UE co-existence for CA(intra-band contiguous DL CA and UL CA) | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125890 | 0821 | - | Addition of Band 28 to 6.6.3.2 Spurious emission band UE co-existence | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125891 | 0822 | - | Addition of Band 28 to 6.6.3.3 Additional spurious emissions | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125892 | 0823 | - | UE output power definition for UL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125893 | 0824 | - | Update TC 6.2.4B Additional maximum power reduction (A-MPR) for UL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125894 | 0825 | - | Update TC 6.5.2B.1 Error vector magnitude (EVM) for UL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125895 | 0826 | - | Update TC 6.2.5B Configured transmitted output power for UL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125896 | 0827 | - | Update TC 7.3B Reference sensitivity level for UL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125897 | 0828 | - | Update TC 7.4B Maximum input level for UL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125898 | 0829 | - | Update TC 7.5B Adjacent Channel Selectivity (ACS) for UL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125899 | 0830 | - | Update TC 7.6.1B In-band blocking for UL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125900 | 0831 | - | Update TC 7.6.2B Out-of-band blocking for UL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125901 | 0832 | - | Update TC 7.6.3B Narrow band blocking for UL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125902 | 0833 | - | Update TC 7.7B Spurious response for UL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125903 | 0834 | - | Update TC 7.8.1B Wide band intermodulation for UL-MIMO | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125906 | 0835 | - | New RF TC for 36.521-1_8.2.1.2.3_Enhanced ICIC for LTE | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125907 | 0836 | - | New RF TC for 36.521-1_8.4.1.2.3_Enhanced ICIC for LTE | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125908 | 0837 | - | TDD CQI Reporting under AWGN conditions-PUCCH 1-0-Demod subframe overlaps with aggressor cell ABS (eICIC) | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125909 | 0838 | - | TDD PHICH Transmit Diversity 2x2-Demod subframe overlaps with aggressor cell ABS (eICIC) | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125910 | 0839 | - | TDD PDSCH Open Loop Spatial Multiplexing 2x2 Demod subframe overlaps with aggressor cell ABS (eICIC) | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125924 | 0841 | - | CR to 8.2.1.1.1_A: Correction to an average throughput measurement | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125925 | 0842 | - | CR to 8.2.1.3.1_A: Correction to an average throughput measurement | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125931 | 0844 | - | Additions to test description TC 6.2.5A.1 and 6.2.5A.2 including CA_1A-19A and CA_1A-21A updates | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125932 | 0845 | - | Updates for TC7.3A: Reference sensitivity level for CA including CA_41C, CA_1-19 and CA_1-21 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125934 | 0846 | - | Update of RF TC for 36.521-1 _ 6.3.5A.1.1 Power Control Absolute power tolerance for CA | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125935 | 0847 | - | Updates for TC6.5.1A: Frequency error for CA | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-126043 | 0849 | - | Updates for 5.2A operating bands for CA_7, CA_38, CA_41, CA_1-18, CA_1-19, CA_1-21, CA_2-17, CA_3-5, CA_3-7, CA_3-8, CA_3-20, CA_4-12, CA_4-13, CA_4-17, CA_4-20 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-126044 | 0850 | - | Updates for 5.4.2A channel bandwidth for CA_7, CA_38, CA_41, CA_1-18, CA_1-19, CA_1-21, CA_2-17, CA_3-5, CA_3-7, CA_3-8, CA_3-20, CA_4-12, CA_4-13, CA_4-17, CA_4-20 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-126063 | 0840 | - | Update of Perf 8.7.1.1 and 8.7.2.1 of 36.521-1 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-126065 | 0843 | - | Clarification of RB position in 9.4.1 and 9.4.2 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-126066 | 0848 | - | Update of minimum test time in Annex G.3.5 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-124010 | 0851 | - | RF Cl 8.1.1: Update of the CA capability table | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-124013 | 0852 | - | RF Cl 8.7.2.1A: Update of test structure according to the work plan | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-124082 | 0853 | - | Correction to TC 7.5.1 Adjacent Channel Selectivity (ACS) | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-124116 | 0854 | - | Addition to test description for SEM for CA | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-124123 | 0855 | - | Addition to minimum requirement for TC 6.5.2A.3.1 in-band emissions for non allocated RB for CA | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-124125 | 0856 | - | TC 6.2.2A.1 UE MOP for CA updates | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-124126 | 0857 | - | CR to Annex G.3A.5: Minimum number of samples for CA performance tests | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-124131 | 0858 | - | Additions to test case for TC 6.2.3A.1 Maximum Power Reduction (MPR) for CA (intra-band contiguous DL CA and UL CA) | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-124133 | 0859 | - | Introduction to TC 6.6.3.3A.1 Additional spurious emissions for CA (intra-band contiguous DL CA and UL CA) | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-124139 | 0860 | - | TC 6.3.2A.1 Minimum Output Power for CA introduction | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-124149 | 0861 | - | RF TC 8.2.2.1.1_A: Adjusting test procedure and requirements to completed Annex G.3A | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-124160 | 0862 | - | TC 7.6.1A.x In-band blocking for CA introduction | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-124163 | 0863 | - | TC 7.6.2A.x Out-of-band blocking for CA introduction | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-124164 | 0864 | - | TC 7.6.3A.x Narrow-band blocking for CA introduction | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-124165 | 0865 | - | TC 7.7A.x Spurious response for CA introduction | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-124166 | 0866 | - | Addition of Receiver test case - Wideband intermodulation for CA | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-124186 | 0867 | - | RF: Addition of references to connection diagrams in CA tests | 10.3.0 | 10.4.0 |
| 2013-03 | RAN#59 | R5-130092 | 0851 | - | Updates of 6.6.3.3 - Addition of Low-channel Band 1 coexistence with PHS condition | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130105 | 0854 | - | Update of TC6.6.3B.2 for Spurious emissions band UE co-existence for UL-MIMO | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130125 | 0856 | - | Update of TC6.6.3B.3 for Additional spurious emissions for UL-MIMO | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130128 | 0857 | - | Update TC 6.2.4B Additional Maximum Power Reduction (A-MPR) for UL-MIMO | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130129 | 0858 | - | Update TC 6.3.5B.1 Power control absolute power tolerance for UL-MIMO | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130130 | 0859 | - | Update TC 6.3.5B.2 Power Control Relative power tolerance for UL-MIMO | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130131 | 0860 | - | Update TC 6.3.5B.3 Aggregate power control tolerance for UL-MIMO | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130132 | 0861 | - | Update TC 6.5.2B.2 Carrier leakage for UL-MIMO | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130133 | 0862 | - | Update TC 6.3.2B Minimum Output Power for UL-MIMO | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130134 | 0863 | - | Update TC 6.6.1B Occupied bandwidth for UL-MIMO | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130135 | 0864 | - | Update TC 6.7B Transmit intermodulation for UL-MIMO | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130176 | 0869 | - | Introduction of new rel-10 Reporting of RI test cases into annexes | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130182 | 0870 | - | CA RF: Correction to Annex I | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130183 | 0871 | - | UL-MIMO RF: Correction to Annex I | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130184 | 0872 | - | eICIC RF: Correction to Annex I | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130185 | 0873 | - | eDL-MIMO RF: Correction to Annex I | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130188 | 0874 | - | RF: Correction to TC 8.2.1.1.1 - Frequency range for partial allocation | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130245 | 0877 | - | Additions to TC 6.6.2.3A.1 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130273 | 0880 | - | Corrections to Annex A DL Reference Measurement and PMI Accuracy Measurements | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130285 | 0881 | - | Correction to TC 8.3.2.1.3 TDD PDSCH Single-layer Spatial Multiplexing on antenna port 7 or 8 with a simultaneous transmission | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130288 | 0882 | - | Addition to TC 9.2.2.2_D TDD CQI Reporting under AWGN conditions - PUCCH 1-1 for eDL-MIMO | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130291 | 0883 | - | Addition to TC 9.3.2.2.2_D TDD CQI Reporting under fading conditions " PUCCH 1-1 for eDL-MIMO | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130292 | 0884 | - | Additions to TC 9.4.2.1.1_D FDD PMI Reporting - PUSCH 1-2 (Multiple PMI) for eDL-MIMO | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130293 | 0885 | - | Additions to TC 9.4.2.1.2_D TDD PMI Reporting - PUSCH 1-2 (Multiple PMI) for eDL-MIMO | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130298 | 0886 | - | CA RF: Correction to TC 7.6.1A | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130299 | 0887 | - | Uncertainties and Test Tolerances for Test case 7.4A | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130308 | 0888 | - | Corrections to test procedure of TC 6.2.2 UE Maximum Output Power | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130309 | 0889 | - | Corrections to test purpose of TC 6.2.3 Maximum Power Reduction (MPR) | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130310 | 0890 | - | Correction to test procedure of TC 6.7A.1 Transmit Intermodulation for CA | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130407 | 0894 | - | Correction to test requirement of 6.2.4A | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130408 | 0895 | - | Correction to test procedure in 6.3.5A.1 | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130411 | 0896 | - | Correction to test procedure in 6.6.2.3A | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130413 | 0897 | - | Correction to 7.6.3A.3 test procedure | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130415 | 0898 | - | Correction to 6.6.3.2A and 6.6.3.3A | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130416 | 0899 | - | Correction to 6.6.3B.2 and 6.6.3B.3 | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130419 | 0900 | - | Correction to 9.3.4 UE-selected subband CQI | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130421 | 0901 | - | Correction to wideband CQI-samples | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130427 | 0902 | - | Update of minimum test time in Annex G.5.4 | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130471 | 0905 | - | Update of spurious emission test cases for introduction of Japanese Regulatory Requirements for LTE Band 8 | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130779 | 0875 | 1 | CA RF: Clarification on Cell ID | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130780 | 0903 | 1 | Correction to 941 and 942 | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130801 | 0908 | - | Additions of CA RF test cases to Annex F | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130804 | 0909 | - | Correction to 6.6.3.2 and 6.6.3.3 | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130806 | 0911 | - | Update TC 6.6.3.2 Spurious emission band UE co-existence | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130808 | 0912 | - | CA RF - SCC configuration references | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130809 | 0913 | - | CA RF Rx - Update of blocking characteristics | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130811 | 0915 | - | LTE RF - Correction to LTE Rx TCs UL allocation location | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130901 | 0916 | - | Update of TC 8.7.2.1_A.1 TDD sustained data rate performance for CA (intra-band contiguous DL CA) | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130902 | 0917 | - | Editorial correction for ON/OFF time mask for CA | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130903 | 0918 | - | Correction to Band 12 frequency for CA_4-12 | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130908 | 0920 | - | Additions of eDL-MIMO to Annex A.4 CQI Reference Measurement Channels | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130909 | 0921 | - | Introduction of TC 8.3.1.1.1_D FDD Single-Layer Spatial Multiplexing on antenna ports without a simultaneous transmission for eDL-MIMO | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130910 | 0922 | - | Introduction of FDD TC 8.3.1.1.2_D FDD Single-Layer Spatial Multiplexing on antenna ports with a simultaneous transmission for eDL-MIMO | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130911 | 0923 | - | Additions to TC 8.3.1.2.1_D FDD PDSCH Dual-layer Spatial Multiplexing for eDL-MIMO | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130912 | 0924 | - | Additions to TC 8.3.2.2.1_D TDD PDSCH Dual-layer Spatial Multiplexing for eDL-MIMO | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130913 | 0925 | - | Additions to TC 8.3.2.1.2_D TDD Single-Layer Spatial Multiplexing on antenna ports without a simultaneous transmission for eDL-MIMO | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130914 | 0926 | - | Additions to TC 8.3.2.1.3_D TDD Single-Layer Spatial Multiplexing on antenna ports with a simultaneous transmission for eDL-MIMO | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130924 | 0934 | - | Update RF TC for 36.521-1_8.2.1.2.3_C FDD PDSCH Transmit div 2x2 Enhanced ICIC for LTE | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130925 | 0935 | - | Update RF TC for 36.521-1_8.4.1.2.3_C FDD PCFICH PDCCH Transmit div 2x2 Enhanced ICIC for LTE | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130935 | 0937 | - | SNR uncertainty | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130936 | 0938 | - | Introduction of new rel-10 Reporting of RI test cases | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130937 | 0939 | - | RF: Corrections to Annex H - Default uplink settings | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130938 | 0940 | - | Update of TC 8.7.2.1_1 TDD sustained data rate performance (Rel-10 and forward) | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130939 | 0941 | - | Clarification to Soft Channel Bit size used by SS and UE | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130940 | 0942 | - | Correction to sustained data rate performance test | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130941 | 0943 | - | Clean up on performance tests | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130944 | 0944 | - | Update TC 6.3.5A.3.1 Aggregate power control tolerance for CA (intra-band contiguous DL CA and UL CA) | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130945 | 0945 | - | Update TC 6.6.3.2A.1 Spurious emission band UE co-existence for CA (intra-band contiguous DL CA and UL CA) | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130946 | 0946 | - | Additions to TC 6.2.5A.1 Configured UE transmitted Power for CA (intra-band contiguous DL CA and UL CA) | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130947 | 0947 | - | Additions to TC 6.6.3.3A.1 Additional spurious emissions for CA (intra-band contiguous DL CA and UL CA) | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130948 | 0948 | - | Additions to TC 7.5A.1 Adjacent Channel Selectivity (ACS) for CA (intra-band contiguous DL CA and UL CA) | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130949 | 0949 | - | Additions to TC 7.5A.2 Adjacent Channel Selectivity (ACS) for CA (intra-band contiguous DL CA without UL CA) | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130950 | 0950 | - | Additions to TC 7.5A.3 Adjacent Channel Selectivity (ACS) for CA (inter-band contiguous DL CA without UL CA) | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130951 | 0951 | - | Correction to test point of Frequency Error for CA and addition of TC for intra-band contiguous DL CA without UL CA | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130961 | 0955 | - | Channel matrix impairments for eDL-MIMO CSI tests | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130965 | 0957 | - | TC 9.4.1.1.1_D FDD Reporting of PMI - PUSCH 3-1 (Single PMI) for eDL-MIMO | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130966 | 0958 | - | TC 9.4.1.1.2_D TDD Reporting of PMI -PUSCH 3-1 (Single PMI) for eDL-MIMO | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130967 | 0959 | - | Additions of eDL-MIMO to Annex C Measurement of Performance Requirements | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130968 | 0960 | - | Additions of eDL-MIMO test cases to Annex F | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130969 | 0961 | - | Additions to Annex G for eDL-MIMO | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130970 | 0962 | - | Addition to TC 9.2.2.1_D FDD CQI Reporting under AWGN conditions - PUCCH 1-1 for eDL-MIMO | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130971 | 0963 | - | Addition to TC 9.3.2.2.1_D FDD CQI Reporting under fading conditions - PUCCH 1-1 for eDL-MIMO | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130972 | 0964 | - | Additions to TC 9.5.1.1_D FDD RI Reporting for eDL-MIMO | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130973 | 0965 | - | Additions to TC 9.5.1.2_D TDD RI Reporting for eDL-MIMO | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130978 | 0966 | - | Correcting TS 36.521-1 to reflect changes in 36.101 | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130979 | 0967 | - | Correction of RF conformance Test case: 8.2.1.3.3 | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130980 | 0968 | - | Addition new TC 8.2.2.3.3 TDD PDSCH Open Loop Spatial Multiplexing 2x2-Demod subframe overlaps with aggressor cell ABS (eICIC) | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130994 | 0969 | - | Corrections to 7.8.1A Wideband intermodulation for CA | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130998 | 0970 | - | TC 9.3.1.2.1_D FDD CQI Reporting under fading conditions - PUSCH 3-1 for eDL-MIMO | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130999 | 0971 | - | TC 9.3.1.2.2_D TDD CQI Reporting under fading conditions - PUSCH 3-1 for eDL-MIMO | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130093 | 0852 | - | Updates of 5.2A Operating bands for CA_11-18 | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | R5-130094 | 0853 | - | Updates of 5.4.2A Channel bandwidth combination for CA_11-18 | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | R5-130124 | 0855 | - | Additions to the inter-band relaxation term for CA 3A-8A in Table 6.2.5A.1.3-3 | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | R5-130140 | 0865 | - | Update TC 7.3A.3 Reference sensitivity level for CA_3-7 and CA_7-20 (inter-band DL CA without UL CA) | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | R5-130141 | 0866 | - | Update TC 7.6.2A.1 Out-of-band blocking for CA_38 (intra-band contiguous DL CA and UL CA) | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | R5-130142 | 0867 | - | Update TC 6.2.2A.1 UE Maximum Output Power for CA_38 (intra-band contiguous DL CA and UL CA) | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | R5-130144 | 0868 | - | Update TC 6.2.4A.1 Additional Maximum Power Reduction (A-MPR) for CA_38 (intra-band contiguous DL CA and UL CA) | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | R5-130239 | 0876 | - | Updates of 6.2.5A - inter-band relaxation term for CA_1A-18A and CA_11A-18A | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | R5-130257 | 0878 | - | Addition of TDD operating band 44 in receiver characteristic test cases | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | R5-130258 | 0879 | - | Addition of TDD operating band 44 in transmitter characteristic test cases | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | R5-130329 | 0891 | - | Updates for 7.6.1A.1 in-band blocking for CA_7 and CA_41 | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | R5-130330 | 0892 | - | Updates for 7.6.2A.1 Out-of-band blocking for CA_7 and CA_41 | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | R5-130348 | 0972 | - | Correction to Spurious emission band UE co-existence and Additional spurious emissions for Band 28 | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | R5-130349 | 0893 | - | Addition of Band 28 to MPR and A-MPR | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | R5-130769 | 0907 | - | Updates of Refsens for inter-band CA test points for CA_1A-19A and CA_1A-21A | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | R5-130783 | 0906 | - | Update TC 6.2.3A.1 Maximum Power Reduction (MPR) for CA_38 (intra-band contiguous DL CA and UL CA) | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | R5-130805 | 0910 | - | Update TC 7.3A.1 Reference sensitivity level for CA_38, CA_3-7 and CA_7-20 (intra-band contiguous DL CA and UL CA) | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | R5-130810 | 0914 | - | CA_2-17 and CA_4-17 addition to 36.521-1 Rx characteristics | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | R5-130905 | 0919 | - | Addition of TDD operating band 44 in chapter 5 | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | R5-130915 | 0927 | - | Addition of band 38 and band 41 in intra-band contiguous CA transmitter test cases | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | R5-130916 | 0928 | - | Update TC 7.6.1A.1 In-band blocking for CA_38 (intra-band contiguous DL CA and UL CA) | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | R5-130917 | 0929 | - | Addition of Inter Band CA combo CA_2A-29A | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | R5-130918 | 0930 | - | Update of 6.2.5A Configured transmitted power for CA | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | R5-130919 | 0931 | - | CA_2-17 and CA_4-17 addition to 36.521-1 Tx characteristics | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | R5-130920 | 0932 | - | Adding transmit power relaxation value for inter-band CA configurations CA_4A-5A and CA_4A-13A | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | R5-130921 | 0933 | - | Adding refsens relaxation value for inter-band CA configurations CA_4A-5A and CA_4A-13A | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | R5-130926 | 0936 | - | Updates for 6.6.2.1A Spectrum emission mask for CA for CA_7 and CA_41 | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | R5-130956 | 0952 | - | Updates for 7.3A.1 Reference sensitivity level for CA_7C | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | R5-130957 | 0953 | - | Updates for 6.2.2A UE Maximum Output Power for CA_7 and CA_41 | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | R5-130958 | 0954 | - | Updates of 7.3A - Reference sensitivity level for CA_11A-18A | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | R5-130964 | 0956 | - | Adding Band 27 to TS 36.521-1 | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | - | - | - | Correction of missing small changes of R5-130806 in Table 6.6.3.2.3-1, 6.6.3.2.5-1, 6.6.3.2_1.3-1 and 6.6.3.2_1.5-1. | 11.0.0 | 11.0.1 |
| 2013-06 | RAN#60 | R5-131147 | 0974 | - | Updates to Chapter7 for Inter Band CA combo CA_2A-29A | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131149 | 0975 | - | New chap8 TDD TC 8.2.2.2.3_C for eICIC | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131150 | 0976 | - | New chap 8 TDD TC 8.4.2.2.3_C for eICIC | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131153 | 0977 | - | Introduction of new rel-11 Reporting of RI test cases | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131154 | 0978 | - | Introduction of new rel-11 Reporting of RI test cases into annexes | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131156 | 0979 | - | Correction to TC 8.4.1.1 test procedure | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131158 | 0980 | - | Introduction of Maximum Input Level test case for CA (inter-band DL CA without UL CA) into annexes | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131210 | 0981 | - | Correction of test applicability for TC 8.2.1.1.1_1: TC 8.2.1.2.1_1 and TC 8.3.2.1.1_1 in 36.521-1 | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131222 | 0982 | - | Update of TC6.6.3B.1 for Transmitter Spurious emissions for UL-MIMO | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131226 | 0983 | - | Clarification to test procedure on applying timing offset between CCs for Chap8 inter-band CA test cases | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131238 | 0984 | - | Update TC 6.2.4A.1 Additional Maximum Power Reduction (A-MPR) for CA_38C (intra-band contiguous DL CA and UL CA) | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131241 | 0985 | - | Update TC 7.3A.1 Reference sensitivity level for CA_4A-12A (intra-band contiguous DL CA and UL CA) | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131242 | 0986 | - | Update TC 7.3A.3 Reference sensitivity level for CA_4A-12A (inter-band DL CA without UL CA) | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131245 | 0987 | - | Update TT of 6.3.5A.3.1 Aggregate power control tolerance for CA (intra-band contiguous DL CA and UL CA) | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131289 | 0988 | - | Uncertainties and Test Tolerances for CA spectrum emission mask | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131358 | 0989 | - | RF: Corrections to RMC-s for sustained data rate test | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131379 | 0990 | - | CA RF - Corrections to 6.2.4A.1 requirements | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131381 | 0991 | - | LTE RF - Editorial correction to 8.2.2.4.2_1 | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131415 | 0992 | - | Updates for 6.6.1A Occupied bandwidth for CA_41C | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131450 | 0993 | - | Addition of band44 in MPR for UL-MIMO | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131473 | 0994 | - | References to wrong section in TS 36.508 | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131518 | 0995 | - | Corrections to Common Test Parameters User-Specific | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131519 | 0996 | - | Corrections to Beamforming Model Parameters | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131521 | 0997 | - | Corrections to Reporting of Channel State Requirements | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131522 | 0998 | - | Corrections to Reporting of Precoding Matrix Indicator Requirements | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131527 | 0999 | - | Additions to Annex G for eDL-MIMO | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131528 | 1000 | - | Additions of eDL-MIMO test cases to Annex F | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131539 | 1001 | - | LTE RF: Minor changes to test case 7.5A.3 | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131540 | 1002 | - | LTE-RF: Clarification to 7.6.1A.3 test procedure | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131541 | 1003 | - | Corrections to TC 6.6.2.3A.1 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131545 | 1004 | - | Additions to TC 9.2.3.1_D for FDD CQI reporting under AWGN conditions - PUCCH 1-1 for eDL-MIMO | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131546 | 1005 | - | LTE-RF: Corrections to CA Open Loop Spatial Multiplexing Performance test cases | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131553 | 1006 | - | Additions to TC 9.2.3.2_D for TDD CQI reporting under AWGN conditions - PUCCH 1-1 for eDL-MIMO | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131555 | 1007 | - | Additions to TC 9.3.1.2.2_D for TDD CQI reporting under fading conditions - PUSCH 3-1 for eDL-MIMO | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131559 | 1008 | - | Additions to TC 9.3.2.2.2_D for TDD CQI reporting under fading conditions - PUCCH 1-1 for eDL-MIMO | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131561 | 1009 | - | Update of 6.2.5A - Delta TIB for CA_11A-18A | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131562 | 1010 | - | Additions to TC 9.4.1.3.2_D for TDD reporting of PMI-PUSCH 3-1 (Single PMI) for eDL-MIMO | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131563 | 1011 | - | Update of 7.3A - Delta RIB for CA_11A-18A | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131567 | 1012 | - | Additions to TC 9.4.2.3.2_D for TDD PMI reporting - PUSCH 1-2 (Multiple PMI) for eDL-MIMO | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131569 | 1013 | - | Additions to TC 9.5.2.2_D for TDD RI reporting - PUCCH 1-1 for eDL-MIMO | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131634 | 1014 | - | Removal of Spurious emission UE co-existence test case 6.6.3.2_1 from annex F | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131649 | 1015 | - | Adding Band28 in some Receiver test cases in section 7 | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131677 | 1016 | - | Correction to the contents of the chapter 5. | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131711 | 1017 | - | Corrections to test procedure of TC 7.5B Adjacent Channel Selectivity (ACS) for UL-MIMO | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131900 | 1018 | - | RF: Correction of Imcs for CQI index 10 (15 RBs) | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131901 | 1019 | - | Corrections to Frequency non-selective scheduling mode | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131902 | 1020 | - | Correction to the missing title of Rel-9 close loop tests | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131903 | 1021 | - | Correction to the references of connection diagrams | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131904 | 1022 | - | Maintenance of Band 23 additional regulatory requirements | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131905 | 1023 | - | Realignment of A-MPR Configuration Tables for Band 27 in TS 36.521-1 | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131906 | 1024 | - | Update on General section of Tx and Rx for Cell IDs | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131907 | 1025 | - | LTE RF: Initial conditions update for test 6.2.5A.2 | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131908 | 1026 | - | CA RF: Alignment for inter-band Rx tests with DL tested only as S-Cell | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131909 | 1027 | - | 36.521-1: Inter-band CA operating bands update | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131911 | 1028 | - | Updates for 7.7A.1 Spurious response for CA | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131915 | 1029 | - | Additions to TC 9.3.1.2.1_D for FDD CQI reporting under fading conditions - PUSCH 3-1 for eDL-MIMO | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131916 | 1030 | - | Additions to TC 9.3.2.2.1_D for FDD CQI reporting under fading conditions - PUCCH 1-1 for eDL-MIMO | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131917 | 1031 | - | Additions to TC 9.4.1.3.1_D for FDD reporting of PMI - PUSCH 3-1 (Single PMI) for eDL-MIMO | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131918 | 1032 | - | Additions to TC 9.4.2.3.1_D for FDD PMI reporting - PUSCH 1-2 (Multiple PMI) for eDL-MIMO | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131919 | 1033 | - | Additions to TC 9.5.2.1_D for FDD RI reporting - PUCCH 1-1 for eDL-MIMO | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131920 | 1034 | - | Adding Band28 in some Transmitter test cases in section 6 | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131921 | 1035 | - | New RF TC for 36.521-1_8.5.1.2.3_C.1_Enhanced ICIC for LTE | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131929 | 1036 | - | Update TT of 6.2.2B_6.2.3B_6.2.4B and 6.2.5B for UL-MIMO | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131930 | 1037 | - | Update TT of 6.3.2B_6.3.3B and 6.3.4B for UL-MIMO | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131931 | 1038 | - | Update TT of 6.3.5B.1_6.3.5B.2 and 6.3.5B.3 for UL- MIMO | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131932 | 1039 | - | Update TT of 6.5.1B_6.5.2B.1_6.5.2B.2_6.5.2B.3_6.5.2B.4 for UL-MIMO | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131933 | 1040 | - | Update TT of 6.6.1B Occupied bandwidth for UL-MIMO | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131934 | 1041 | - | Update TT of 6.6.2.1B_6.6.2.2B and 6.6.2.3B for UL- MIMO | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131935 | 1042 | - | Update TT of 6.7B Transmit intermodulation for UL-MIMO | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131936 | 1043 | - | Update TT of 6.8B Time alignment error for UL-MIMO | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131937 | 1044 | - | Update TT of 7.3B Reference sensitivity level for UL-MIMO | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131952 | 1045 | - | Corrections to TC 6.2.5A.1 Configured UE transmitted Power for CA (intra-band contiguous DL CA and UL CA) | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131954 | 1046 | - | Maintenance of Band 23 Spurious Emissions Requirements | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131955 | 1047 | - | Corrections to Demodulation Requirements | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131956 | 1048 | - | Updates to sustained data rate test case | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131957 | 1049 | - | New RF TC for 36.521-1_9.2.1.3_C.1_Enhanced ICIC for LTE | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131958 | 1050 | - | RF: Structure clean-up for chapter 9 (including eICIC and eDL-MIMO tests) | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131959 | 1051 | - | Update TT of 7.4B_7.5B_7.6.1B_7.6.2B_7.6.3B_7.7B and 7.8.1B for UL-MIMO | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131960 | 1052 | - | Update of TC6.6.3B.3 for Additional spurious emissions for UL-MIMO | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131961 | 1053 | - | Update of TC6.6.3B.2 for Spurious emissions band UE co-existence for UL-MIMO | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131962 | 1054 | - | Updates of 6.2.2.3 - maximum output power for UE that supports both Band 18 and 26 | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131963 | 1055 | - | Update of 6.2.4 Test points for A-MPR when NS_05 is signalled | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131964 | 1056 | - | Updates of 7.3.3 - Refsens for UE that supports both Band 18 and Band 26 | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131969 | 1057 | - | CA RF: Alignment for tests with PCC and SCC switching | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131971 | 1059 | - | Introduction of Maximum Input Level test case for CA (inter-band DL CA without UL CA) | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131972 | 1060 | - | Addition of Maximum input level for inter-band CA and updates to test configuration for some Rx TCs | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131973 | 1061 | - | Updates to FDD sustained data rate test case for CA | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131974 | 1062 | - | Clarification on Time offset between cells | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131975 | 1063 | - | Update TC 6.2.3A.1 Maximum Power Reduction (MPR) for CA (intra-band contiguous DL CA and UL CA) | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131976 | 1064 | - | Update of UE co-existence requirement for CA | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131977 | 1065 | - | Update TC 6.2.5A.1 Configured UE transmitted Output Power for CA_3A-7A, CA_4A-12A and CA_7A-20A (intra-band contiguous DL CA and UL CA) | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131978 | 1066 | - | Updates for 6.2.4A A-MPR for CA_41C | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131979 | 1067 | - | Updates for 7.5A.1 Adjacent Channel Selectivity (ACS) for CA_41C | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131980 | 1068 | - | Updates for 7.6.1A.1 in-band blocking for CA_41C | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131981 | 1069 | - | Updates for 7.6.2A.1 Out-of-band blocking for CA_41C | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131982 | 1070 | - | Updates for 7.6.3A.1 Narrow band blocking for CA_41C | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131983 | 1071 | - | Updates for 7.8.1A.1 Wideband intermodulation for CA_41C | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131985 | 1072 | - | Update of CA refsens test 7.3A for inter-band CA configurations CA_4A-5A and CA_4A-13A | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131987 | 1073 | - | Addition of band 44 in MPR | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131990 | 1074 | - | CA_1A-18A, CA_2A-17A, CA_3A-8A, CA_4A-5A, CA_4A-13A, CA_4A-17A, CA_11A-18A addition and corrections to Reference Sensitivity TC 7.3A | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-132079 | 1058 | - | Corrections to TC 6.6.3.3A.1 Additional spurious emissions for CA (intra-band contiguous DL CA and UL CA) | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-132080 | 1075 | - | CA RF - Corrections to Blocking Characteristics in 7.6 | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-132103 | 1076 | - | Change Spurious emission UE co-existence test cases to align with release independence rule | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-132105 | 1077 | - | CA RF: Corrections to message contents with regard to transmission mode | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-132110 | 1078 | - | Updated test points test case 6.2.3 A-MPR for NS_15 band26 | 11.0.1 | 11.1.0 |
| 2013-06 | RAN#60 | R5-132113 | 1079 | - | New test cases for LTE B14 public safety high power UE | 11.0.1 | 11.1.0 |
| 2013-09 | RAN#61 | R5-133070 | 1080 | - | Correction to test points in Additional Spurious Emissions Coexistence test case | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133073 | 1081 | - | LTE Type A performance requirements - Changes to clause 3 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133074 | 1082 | - | LTE Type A performance requirements - Changes to clause 8.1.1 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133075 | 1083 | - | LTE Type A performance requirements - Changes to Annex A | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133076 | 1084 | - | LTE Type A performance requirements - Introduction of Annex B.5 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133077 | 1085 | - | LTE Type A performance requirements - Introduction of the new test case 9.3.5.1.1. | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133232 | 1086 | - | Corrections to CA TC 6.5.2A.3 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133247 | 1087 | - | Addition of Band 31 to 36.521-1 General s05 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133248 | 1088 | - | Addition of Band 31 to 36.521-1 RX s07 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133249 | 1089 | - | Addition of Band 31 to 36.521-1 TX s06 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133252 | 1090 | - | RF: Minor correction to test points for spurious emissions UE co-existence | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133258 | 1091 | - | Update TC 7.3A.3 Reference sensitivity level for CA_5A-12A (inter-band DL CA without UL CA) | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133261 | 1092 | - | Update the minimum requirements of TCs 7.6.3A.1, 7.7A.1 and 7.8.1A.1 for CA | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133262 | 1093 | - | Update TC 6.6.3B.2 of Spurious emission band UE co-existence for UL-MIMO | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133263 | 1094 | - | Update TCs 6.2.2B and 6.2.5B for UL-MIMO | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133265 | 1095 | - | RF: Addition of missing UL-RMC-s for Tx testing | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133266 | 1096 | - | Correction of TCs 7.3 and 7.3B Reference sensitivity level for Band 27 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133268 | 1097 | - | CA RF: Addition of new CQI tests for CA | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133298 | 1098 | - | Correction to Spurious emission band UE co-existence | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133415 | 1099 | - | Correction to the repeat steps in 7.6.1A, 7.6.3A and 7.8.1A | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133418 | 1100 | - | Correction to CA sustained data rate performance test | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133423 | 1101 | - | Correction to eICIC Performance test cases | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133425 | 1102 | - | Correction to UE category in Tx test cases | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133426 | 1103 | - | Correction to Spurious emission band UE co-existence | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133431 | 1104 | - | Correction to MBMS Performance test cases | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133474 | 1105 | - | Update TC of 8.7.2.1_A.1 TDD sustained data rate performance for CA (intra-band contiguous DL CA) | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133517 | 1106 | - | Adding the minimum channel spacing for intra-band non-contiguous CA | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133522 | 1107 | - | Corrections to Annex F for Maintenance of Band 23 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133523 | 1108 | - | Corrections to description for definition of MIMO Correlation Matrices using cross polarized antennas | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133524 | 1109 | - | Corrections to UE maximum output power for CA | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133525 | 1110 | - | Corrections to General Maintenance of Operating Bands and Channel Arrangements | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133532 | 1111 | - | Introduction of eICIC_enh_LTE to Annex C | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133534 | 1112 | - | Corrections to TC 6.6.2.3A.1 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133535 | 1113 | - | Corrections to TC 7.5A.1 Adjacent Channel Selectivity (ACS) for CA (intra-band contiguous DL CA and UL CA) | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133800 | 1114 | - | Correction to test frequency in Additional Spurious Emissions test cases for NS_15 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133801 | 1115 | - | Update applicability of test cases 9.4.2.2.1 and 9.4.2.2.2 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133802 | 1116 | - | Correction to applicability for TDD-TM8 test cases | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133803 | 1117 | - | Editorial correction to 6.6.3.3 Additional spurious emissions | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133804 | 1118 | - | Correction the Minimum conformance requirements for test case 6.2.2B | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133805 | 1119 | - | CA RF: Adding missing DL-RMC-s | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133806 | 1120 | - | Update TC of 8.2.1.1 FDD PDSCH Single Antenna Port Performance (Cell-Specific Reference Symbols) for CA | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133807 | 1121 | - | Update TC of 8.2.1.4 FDD PDSCH Closed Loop Spatial Multiplexing Performance (Cell-Specific Reference Symbols) for CA | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133808 | 1122 | - | Addition of new TC 8.2.1.4.2_A.2 for FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (inter band DL CA) | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133811 | 1123 | - | Update of CA TC 7.3A.1 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133812 | 1124 | - | Update of CA TC 7.4A.1 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133819 | 1125 | - | Correction of applicability for FDD RF TCs 9.3.4.1.1, 9.3.4.2.1 & 9.4.1.2.1and TDD RF TCs 9.3.4.1.2, 9.3.4.2.2 & 9.4.1.2.2 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133823 | 1126 | - | Introduction of eICIC_enh_LTE to Annex A to reference measurement channels for PDCCH/PCFICH | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133824 | 1127 | - | Introduction of eICIC_enh_LTE to Annex A DL reference measurement channels | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133828 | 1128 | - | Corrections to 6.2.4 A-MPR for Maintenance of Band 23 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133829 | 1129 | - | Corrections to Transmitter Characteristics Maintenance | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133830 | 1130 | - | Alignment of test configuration tables (test frequencies) in Additional Spurious Emissions tests cases | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133831 | 1131 | - | Corrections to Receiver Characteristics Maintenance | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133832 | 1132 | - | Corrections to 6.6.2.2 Additional Spectrum Emission Mask for Maintenance of Band 23 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133833 | 1133 | - | Updates to Chapter7 for Inter Band CA combo CA_4A-29A | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133838 | 1134 | - | Update Minimum Requirements of TCs 6.2.4 and 6.2.5 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133845 | 1135 | - | Correction to Configured UE transmitted output power for CA | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133846 | 1136 | - | Corrections to TC 6.2.5A.2 Configured UE transmitted Power for CA (inter-band DL CA without UL CA) | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133847 | 1137 | - | Updates to Wideband intermodulation for CA test cases | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133850 | 1138 | - | Correction to measurement target of CA Rx test cases | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133851 | 1139 | - | Corrections to TC 7.5A.2 Adjacent Channel Selectivity (ACS) for CA (intra-band contiguous DL CA without UL CA) | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133852 | 1140 | - | Update TC of 8.7.1.1_A.1 FDD Sustained data rate performance for CA (intra band contiguous DL CA) | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133853 | 1141 | - | Update TC of 8.7.1.1_A.2 FDD Sustained data rate performance for CA (inter band DL CA) | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133854 | 1142 | - | Updates of Annex for CA test cases | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133863 | 1143 | - | Clarification on Bandwidth Combination Set for Carrier Aggregation Transmitter tests | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133865 | 1144 | - | Update TC 6.2.5A.1 Configured UE transmitted Output Power for CA_5A-12A (intra-band contiguous DL CA and UL CA) | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133866 | 1145 | - | Update the minimum requirements of TCs 7.5A.1, 7.6.1A.1 and 7.6.2A.1 for CA | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133867 | 1146 | - | Update TC 7.3A.1 Reference sensitivity level for CA (intra-band contiguous DL CA and UL CA) | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133869 | 1147 | - | Addition of Refsens for intra-band non-contiguous CA | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133870 | 1148 | - | Update TC 7.3A.3 reference sensitivity level for CA_3-8 (inter-band DL CA without UL CA) | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133871 | 1149 | - | Test cases and test system uncertainty for LTE B14 public safety highpower UE | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133873 | 1150 | - | Addition of new TC 8.3.1.1.3 for FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 with TM9 Interference Model | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133874 | 1151 | - | Addition of new TC 8.3.2.1.4 for TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 with TM9 Interference Model | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133886 | 1152 | - | eICIC RF: Addition of new RI tests and OCNG patterns for eICIC | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133887 | 1153 | - | Update of CA TC 6.2.3A and TC 6.2.4A | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133888 | 1154 | - | CA RF: Corrections to reference sensitivity requirements for CA | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133889 | 1155 | - | Correction to Test Requirement in 7.3A.3 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133890 | 1156 | - | Update TC of 8.2.1.3 FDD PDSCH Open Loop Spatial Multiplexing Performance (Cell-Specific Reference Symbols)for CA | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133898 | 1157 | - | Clarification on Bandwidth Combination Set for Carrier Aggregation Receiver tests | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133899 | 1158 | - | Addition of CA TC 6.6.2.2A | 11.1.0 | 11.2.0 |
| 2013-12 | RAN#62 | R5-134130 | 1159 | - | RF: Corrections to the notes in the band UE co-existence requirements table | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134131 | 1160 | - | RF: Corrections to sustained data rate test case | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134132 | 1161 | - | RF: Clean-up of uplink reference measurement channels | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134139 | 1162 | - | eICIC RF: Corrections to PHICH demodulation test | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134161 | 1163 | - | Corrections to Transmitter Requirements for Intra-band Non-contiguous CA | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134169 | 1164 | - | Introduction of I.2.6 Tests for feICIC (Further Enhanced Non CA- based ICIC for LTE) | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134170 | 1165 | - | Introduction of TC 8.2.1.2.3_E.1 FDD PDSCH Transmit diversity 2x2 for feICIC (non-MBFSN ABS) | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134171 | 1166 | - | Introduction of TC 8.2.2.2.3_E.1 TDD PDSCH Transmit diversity 2x2 for feICIC (non-MBSFN ABS) | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134172 | 1167 | - | Introduction of TC 8.2.1.4.1_E.1 FDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x2 for feICIC (non-MBSFN ABS) | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134173 | 1168 | - | Introduction of TC 8.2.2.4.1_E.1 TDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x2 for feICIC (non-MBSFN ABS) | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134174 | 1169 | - | Introduction of TC 8.4.1.2.3_E.1 FDD PCFICH/PDCCH Transmit Diversity 2x2 for feICIC (non-MBSFN ABS) | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134175 | 1170 | - | Introduction of TC 8.4.1.2.3_E.2 FDD PCFICH/PDCCH Transmit Diversity 2x2 for feICIC (MBSFN ABS) | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134176 | 1171 | - | Introduction of TC 8.4.2.2.3_E.1 TDD PCFICH/PDCCH Transmit Diversity 2x2 for feICIC (non-MBSFN ABS) | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134177 | 1172 | - | Introduction of TC 8.4.2.2.3_E.2 TDD PCFICH/PDCCH Transmit Diversity 2x2 for feICIC (MBSFN ABS) | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134178 | 1173 | - | Introduction of Demodulation feICIC Test Cases to Annex F | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134179 | 1174 | - | Introduction of TC 9.2.1.5_E.1 FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for feICIC (non-MBSFN ABS) | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134180 | 1175 | - | Introduction of TC 9.2.1.6_E.1 TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for feICIC (non-MBSFN ABS) | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134181 | 1176 | - | Introduction of TC 9.3.1.3.1_E.1 FDD CQI Reporting under fading conditions - PUSCH 3-0 for feICIC (non-MBSFN ABS) | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134182 | 1177 | - | Introduction of TC 9.3.1.3.2_E.1 TDD CQI Reporting under fading conditions - PUSCH 3-0 for feICIC (non-MBSFN ABS) | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134183 | 1178 | - | Introduction of TC 9.5.4.1_E FDD RI Reporting - PUCCH 1-0 for feICIC (non-MBSFN ABS) | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134184 | 1179 | - | Introduction of TC 9.5.4.2_E TDD RI Reporting - PUCCH 1-0 for feICIC (non-MBSFN ABS) | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134185 | 1180 | - | Introduction of Reporting of Channel State Information feICIC Test Cases to Annex F | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134196 | 1181 | - | Corrections to 7.5A.3 Adjacent Channel Selectivity (ACS) for CA (inter-band DL CA without UL CA) | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134199 | 1182 | - | Corrections to MPR for intra-band non-contiguous CA | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134234 | 1183 | - | CA RF: Corrections to requirements for In-Band blocking for Inter-Band without UL-CA | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134247 | 1184 | - | RF: Corrections to requirements for NS_11 (Band 23) | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134250 | 1185 | - | RF: Band edge relaxation for UE-s supporting Band 18 and Band 26 | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134289 | 1187 | - | Correction to Maximum Power Reduction (MPR) for CA Test Procedure | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134413 | 1189 | - | Update TT of TC 8.2.1.4.2_A.2 FDD PDSCH Closed Loop Multi Layer 4x2 for CA | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134414 | 1190 | - | Correction of TC 6.2.4A Test Requirements for CA_38C | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134421 | 1191 | - | The addition of Beamforming model for EPDCCH localized transmission | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134423 | 1192 | - | The addition of Beamforming model for EPDCCH distributed transmission | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134424 | 1193 | - | The addition of Downlink physical setup for EPDCCH | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134431 | 1194 | - | LTE Type A performance requirements - Introduction of the new test case 8.2.1.2.4 | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134432 | 1195 | - | LTE Type A performance requirements - adding TC 8.2.1.2.4 and 9.3.5.1.1 to Annex E and G | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134445 | 1196 | - | TU and TT for 36.521-1 eICIC FDD PCFICH/PDCCH and PHICH demodulation tests | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134477 | 1197 | - | Correction to CA Sustained data rate tests | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134478 | 1198 | - | Correction to CQI-ReportConfig-DEFAULT for CA performance tests | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134485 | 1199 | - | Clarification of UL resource blocks for CA_4A-17A in 7.3A.3 | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134488 | 1200 | - | Correction to test requirement in 6.2.4 | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134489 | 1201 | - | Correction to test requirement in 6.6.3.2 | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134490 | 1202 | - | Additional exception message for SIB3 in MBMS performance test | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134800 | 1204 | - | Introduction of UE TM3 Demodulation Performance under High Speed to Annex F | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134801 | 1205 | - | Introduction of UE TM3 Demodulation Performance under High Speed | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134802 | 1206 | - | Corrections to eDL-MIMO Channel State Information Test | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134805 | 1207 | - | Update Minimum Requirements of TCs 6.6.3.2 and 6.6.3.2A Spurious emission band UE co-existence | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134806 | 1208 | - | Change Spurious emission UE co-existence test case for LTE CA to align with release independence rule | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134807 | 1209 | - | Clarification of applicability of additional relaxation RIB,c on Reference Sensitivity Test Cases | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134816 | 1215 | - | Addition of new Test Case PUCCH 1-1 for Interference rejection | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134817 | 1216 | - | Addition of interference rejection test cases TC 8.2.2.2.4 and TC 8.2.2.4.3 | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134828 | 1217 | - | Clarification of multi-cluster transmission terminology | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134829 | 1218 | - | Corrections to Pcmax | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134830 | 1219 | - | Corrections to test descriptions reference tables for Transmitter Characteristics test cases | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134832 | 1220 | - | Corrections to definition of 5+20 MHz for spectrum emission mask for CA in Annex F | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134857 | 1223 | - | Update TC 6.2.4 A-MPR for NS_19 | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134858 | 1224 | - | Update TC 6.6.3.3 Additional Spurious Emissions for NS_19 | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134859 | 1225 | - | Simplification of test points in Additional Spurious Emissions Coexistence test case | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134861 | 1227 | - | Addition of Sustained data rate test(FDD) for category 6 and 7 UEs | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134865 | 1228 | - | CA RF: Power alignment between uplink component carriers | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134866 | 1229 | - | Completion of test case 6.7A Transmit intermodulation for CA | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134867 | 1230 | - | Update Minimum Requirements of TCs 7.3 and 7.3.1A Reference sensitivity power level | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134868 | 1231 | - | Clean-up of CA Rx test cases | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134871 | 1232 | - | Update Minimum Requirements and TT of TC 8.2.1.3.1_A | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134875 | 1233 | - | Addition of Inter Band CA combo CA_3A-5A, CA_3A-20A | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134876 | 1234 | - | Addition of Inter Band CA combo CA_4A-7A | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134878 | 1235 | - | Correction and applicability clarification to multi-cluster allocations for UL CA TCs 6.2.3A, 6.2.4A, 6.6.2.1A, 6.6.2.2A, 6.6.2.3A and 6.6.3.3A | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134879 | 1236 | - | Clarification of CA Reference sensitivity level | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134880 | 1237 | - | Correction to blocking requirements and use of delta RIB | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134881 | 1238 | - | Addition of new TC 7.4A.4 Maximum input level for CA (intra band non-contiguous DL CA without UL CA) | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134882 | 1239 | - | Addition of new TC 7.5A.4 Adjacent Channel Selectivity (ACS) for CA(intra band non-contiguous DL CA without UL CA) | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134889 | 1244 | - | Uncertainties and Test Tolerances for eICIC PDSCH demodulation tests | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134893 | 1247 | - | Addition of applicabilities of LTE Type A performance requirements | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134894 | 1248 | - | Test Procedure, uncerts and TT for Rx Test cases with intra-band contiguous UL CA | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134976 | 1251 | - | Update TT of TC 8.2.1.4.2_A.1 FDD PDSCH Closed Loop Multi Layer 4x2 for CA | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134977 | 1252 | - | Update of CA CSI tests | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134978 | 1253 | - | Update of eICIC Performance test cases | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134985 | 1254 | - | LTE Type A performance requirements - Changes to TC 9.3.5.1.1 | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134987 | 1255 | - | RF: Corrections to requirements for NS_15 (Band 26) | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134988 | 1256 | - | Corrections to Band 23 NS_11 and NS_20 Test Points | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134995 | 1259 | - | Update on MPR and A-MPR test requirements | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-135012 | 1261 | - | Clarification in regard to 36.521-1 tests not included in 36.521-2 Applicability table | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-135031 | 1222 | - | RF TC Update for 36.521-1_9.2.1.3_C1 eICIC | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-135046 | 1226 | - | RF: Updates to the downlink reference measurement channels | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134277 | 1186 | - | Addition of Rel-12 CA band combinations(CA_3-19 and CA_19-21) to Chap. 5 | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-134339 | 1188 | - | Updates to TC 6.2.2A.1 for CA_3C | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-134540 | 1203 | - | Applicability statement of new 5MHz BB tests for Band 31 in TS 36.521-1 | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-134808 | 1210 | - | Updates of 5.4.2A Channel bandwidth for CA_1A-18A | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-134809 | 1211 | - | Updates of 5.2A Operating bands for CA_1A-26A | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-134810 | 1212 | - | Updates of 5.4.2A Channel bandwidth for CA_1A-26A | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-134811 | 1213 | - | Updates of 6.2.5A Delta TIB for CA_1A-26A | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-134812 | 1214 | - | Updates of 7.3A Delta RIB for CA_1A-26A | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-134833 | 1221 | - | Correction to Tx TCs for lower tolerance relaxation, removal of 6.2.5A.2 and addition of Rel-12 CA band combinations(CA_3-19 and CA_19-21) to 6.2.5 | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-134884 | 1240 | - | Addition of Rel-12 CA band combination(CA_3-19 and CA_19-21) to Refsens | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-134886 | 1241 | - | Updates for 5.2A operating bands and 5.4.2A channel bandwidth for CA_3C | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-134887 | 1242 | - | Updates to TC 6.2.3A.1 for CA_3C | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-134888 | 1243 | - | Updates to TC 6.6.2.1A.1 for CA_3C | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-134891 | 1245 | - | Updates to FDD PDSCH Single Antenna and Transmit Diversity 2x2 performance test cases for 5MHz BW | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-134892 | 1246 | - | Updates to 8.2.1.3.1 for FDD PDSCH Open Loop Spatial Multiplexing 2x2 for 5MHz BW | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-134896 | 1249 | - | Updates to TC 9.2.1.1 FDD CQI reporting under AWGN conditions - PUCCH 1-0 test case for 5MHz BW | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-134897 | 1250 | - | Updates to TC 9.3.2.1.1 and 9.3.2.1.1_1 FDD CQI Reporting under fading conditions - PUCCH 1-0 test case for 5MHz BW | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-134989 | 1257 | - | Corrections to definition of 5+20 MHz for spectrum emission mask for CA | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-134992 | 1258 | - | Updates to 8.5.1.2 FDD PHICH Transmit Diversity Performance tests | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-135068 | 1262 | - | Updates to 8.2.1.4 FDD PDSCH Closed Loop Single/Multiple layer Spatial Multiplexing 2x2 for 5MHz BW | 11.3.0 | 12.0.0 |
| 2014-03 | RAN#63 | R5-140228 | 1264 | - | Uncertainties for eICIC 9.x CSI Test Cases | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140245 | 1265 | - | eDL-MIMO RF: Corrections to 8x2 MIMO tests | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140247 | 1266 | - | CA RF: Corrections to sustained data rate tests | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140248 | 1267 | - | RF: Corrections to OCNG pattern for PMI tests | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140249 | 1268 | - | RF: Corrections to cross references between tests | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140250 | 1269 | - | feICIC RF: Corrections to some subclause-titles | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140261 | 1270 | - | CoMP RF: Introduction of CoMP section in Annex I | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140388 | 1272 | - | LTE Type A performance requirements - Introduction of the new test case 9.3.5.1.2 | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140444 | 1273 | - | Updates of 6.2.5 Delta TIB for CA_1A-26A | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140446 | 1274 | - | Updates of 7.3 Delta RIB for CA_1A-26A | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140448 | 1275 | - | Updates of 6.6.3.2 Spurious emission band UE co-existence for B41 | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140499 | 1276 | - | Update of eICIC Performance tests | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140505 | 1277 | - | Correction to test configuration parameter in Tx | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140508 | 1278 | - | Update of CA FDD and TDD performance tests | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140509 | 1279 | - | Addition of CQI configuration in CA closed loop performance tests | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140511 | 1280 | - | Addition of SCC activation and CQI configuration in CA CQI Reporting tests | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140561 | 1281 | - | Removal of [ ]s in Annex G.6 | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140800 | 1282 | - | Corrections to NS signalling for CA refsens | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140801 | 1283 | - | Correction to statistical testing for Inter-band CA Rx tests | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140802 | 1284 | - | Additions to combinations of channel model parameters | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140803 | 1285 | - | Addition of a Fixed Reference Channel two antenna ports | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140804 | 1286 | - | Corrections to TDD eDL-MIMO test cases | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140805 | 1287 | - | Correction the category information of Table 8.2.1.1.2.3-2 and Table 8.2.1.1.2.5-1 in test case 8.2.1.1.2 | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140806 | 1288 | - | Correction the category information in Table 8.2.1.2.1.3-2 and Table 8.2.1.2.1.3-2 for Test case 8.2.1.2.1 | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140807 | 1289 | - | Correction the test case applicability of test case 8.2.1.3.2 | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140811 | 1290 | - | Addition of A.3.9.4 TDD SDR Reference Channel for EPDCCH | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140812 | 1291 | - | Addition of A.3.9.3 FDD SDR Reference Channel for EPDCCH | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140813 | 1292 | - | Addition of OCNG Pattern for EPDCCH TDD performance requirements | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140814 | 1293 | - | Addition of OCNG Pattern for EPDCCH FDD performance requirements | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140815 | 1294 | - | Addition of Reference Measurement Channels for EPDCCH performance requirements | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140816 | 1295 | - | Introduction of RMCs and power levels for CoMP in annex | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140818 | 1296 | - | Addition of CA_1A-8A to 36.521-1 Chapter 5 | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140819 | 1297 | - | Addition of CA_1A-8A to 36.521-1 Chapter 6 | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140820 | 1298 | - | Addition of CA_1A-8A to 36.521-1 Chapter 7 | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140823 | 1300 | - | Corrections to definition of 20 MHz and 5MHz for transmit intermodulation for CA in 6.7A | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140824 | 1301 | - | Updates to TC 7.4A.1 for CA_3C | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140825 | 1302 | - | Updates to TC 6.6.3.2A for CA_3C | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140826 | 1303 | - | Corrections to definition of 20 MHz and 5MHz for occupied bandwidth for CA in 6.6.1A | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140827 | 1304 | - | Updates to TC 7.3A.1 for CA_3C | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140828 | 1305 | - | Updates to TC 7.3A.2 for CA_3C | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140831 | 1306 | - | Additions to TC 8.2.2.2.3_E.1 TDD PDSCH Transmit diversity 2x2 for feICIC (non-MBSFN ABS) | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140832 | 1307 | - | Additions to TC 8.2.1.4.1_E.1 FDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x2 for feICIC (non-MBSFN ABS) | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140833 | 1308 | - | Additions to TC 8.2.2.4.1_E.1 TDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x2 for feICIC (non-MBSFN ABS) | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140834 | 1309 | - | Additions to TC 8.4.1.2.3_E.1 FDD PCFICH/PDCCH Transmit Diversity 2x2 for feICIC (non-MBSFN ABS) | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140835 | 1310 | - | Additions to TC 8.4.2.2.3_E.1 TDD PCFICH/PDCCH Transmit Diversity 2x2 for feICIC (non-MBSFN ABS) | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140836 | 1311 | - | Additions to TC 8.4.1.2.3_E.2 FDD PCFICH/PDCCH Transmit Diversity 2x2 for feICIC (MBSFN ABS) | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140837 | 1312 | - | Additions to TC 8.4.2.2.3_E.2 TDD PCFICH/PDCCH Transmit Diversity 2x2 for feICIC (MBSFN ABS) | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140838 | 1313 | - | Additions to TC 9.2.1.5_E.1 FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for feICIC (non-MBSFN ABS) | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140839 | 1314 | - | Additions to TC 9.2.1.6_E.1 TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for feICIC (non-MBSFN ABS) | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140840 | 1315 | - | Additions to TC 9.3.1.3.1_E.1 FDD CQI Reporting under fading conditions - PUSCH 3-0 for feICIC (non-MBSFN ABS) | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140841 | 1316 | - | Additions to TC 9.3.1.3.2_E.1 TDD CQI Reporting under fading conditions - PUSCH 3-0 for feICIC (non-MBSFN ABS) | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140842 | 1317 | - | Additions to TC 9.5.4.1_E FDD RI Reporting - PUCCH 1-0 for feICIC (non-MBSFN ABS) | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140843 | 1318 | - | Additions to TC 9.5.4.2_E TDD RI Reporting - PUCCH 1-0 for feICIC (non-MBSFN ABS) | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140844 | 1319 | - | Introduction of feICIC to Annex G | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140845 | 1361 | - | Addition of PHICH demod requirements for fEICIC | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140846 | 1320 | - | Addition to TC 8.2.2.3.3_E.1 TDD PDSCH Open Loop Spatial Multiplexing 2x2 for feICIC (non-MBSFN ABS) | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140847 | 1321 | - | Corrections to test configuration notes of CA soft buffer | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140856 | 1322 | - | Additions to TC 8.2.1.2.3_E.1 FDD PDSCH Transmit diversity 2x2 for feICIC (non-MBFSN ABS) | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140868 | 1323 | - | Uncertainties and Test Tolerances for Interf_Rej Test case 8.2.1.2.4 | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140877 | 1324 | - | Introduction of TC 9.5.5.1_F.1 FDD RI Reporting with Single CSI process | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140878 | 1325 | - | Introduction of TC 9.5.5.2_F.1 TDD RI Reporting with Single CSI process | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140881 | 1326 | - | Addition to TC 8.2.1.3.3_E.1 FDD FDD PDSCH Open Loop Spatial Multiplexing 2x2 for feICIC (non-MBSFN ABS) | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140887 | 1327 | - | Correction to eICIC CQI Reporting tests | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140888 | 1328 | - | eICIC RF: Corrections to requirements, procedure and UL-RMCs for CQI tests | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140889 | 1329 | - | Addition of TC 8.7.3.1 FDD sustained data rate performance for EPDCCH scheduling | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140890 | 1330 | - | Addition of TC 8.8.1.1.1 FDD distributed EPDCCH performance | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140891 | 1331 | - | Addition of TC 8.7.4.1 TDD sustained data rate performance for EPDCCH scheduling | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140892 | 1332 | - | Addition of TC 8.8.1.2.1 TDD distributed EPDCCH performance | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140893 | 1333 | - | Addition of TC 8.8.2.1.1 FDD localized EPDCCH performance | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140894 | 1334 | - | Addition of TC 8.8.2.1.2 FDD EPDCCH localized transmission with TM10 quasi co-location Type-B | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140895 | 1335 | - | Addition of TC 8.8.2.2.1 TDD localized EPDCCH performance | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140896 | 1336 | - | Addition of TC 8.8.2.2.2 TDD EPDCCH localized transmission with TM10 quasi co-location Type-B | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140922 | 1337 | - | Modify Applicability for LTE B14 public safety high power UE test cases | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-141003 | 1338 | - | Addition of FDD CQI Reporting under AWGN conditions - Single CSI Process TC 9.2.4.1 | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-141004 | 1339 | - | Addition of TDD CQI Reporting under AWGN conditions - Single CSI Process TC 9.2.4.2 | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-141006 | 1340 | - | Addition of TC 8.2.1.7 FDD Carrier Aggregation with power imbalance | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-141007 | 1341 | - | Updates to TC 7.4A.2 for CA_3C | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-141008 | 1342 | - | Addition of TC 9.3.6.1_F FDD CQI Reporting under fading conditions multiple CSI processes for CoMP | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-141009 | 1343 | - | Addition of TC 9.3.6.2_F TDD CQI Reporting under fading conditions multiple CSI processes for CoMP | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-141011 | 1345 | - | Corrections to definition of 20 MHz and 5MHz for minimum output power for CA in 6.3.2A | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-141012 | 1346 | - | Addition of Intra NC CA to chap. 5 and related symbol to chap. 3 | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-141016 | 1347 | - | Correction and proposed resolution to TBDs UL CA TCs 6.2.4A and 6.6.3.3A | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-141017 | 1348 | - | Update for CA Rx TCs | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-141018 | 1349 | - | PCC/SCC switching in inter-band CA Rx test cases | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-141026 | 1350 | - | Introduction of new TC 8.3.1.3.2_F FDD PDSCH Performance with DCI format 2D, non Quasi Co-located Antenna Ports, Same Cell ID and multiple NZP CSI-RS resources for CoMP | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-141027 | 1351 | - | Introduction of TC 8.3.1.3.3_F FDD PDSCH Performance with DCI format 2D, non Quasi Co-located Antenna Ports, Different Cell ID, Colliding CRS and single NZP CSI-RS for CoMP | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-141028 | 1352 | - | Addition of new demod scenario for DL COMP - Same Cell ID and single NZP CSI-RS resource | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-141029 | 1353 | - | Corrections to TM9 test to verify correct SNR estimation | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-141038 | 1354 | - | Correction to sustained downlink data rate tests | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-141040 | 1355 | - | Changes to test frequencies in Tx test cases for Band 28 | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-141041 | 1356 | - | Updates of Reference sensitivity for Intra-band non-contiguous CA | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-141042 | 1357 | - | Update TC of 7.4A.4 Maximum input level for CA (intra band non-contiguous DL CA without UL CA) | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-141043 | 1358 | - | Update TC of 7.5A.4 Adjacent Channel Selectivity (ACS) for CA (intra band non-contiguous DL CA without UL CA) | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-141044 | 1359 | - | Corrections to maximum output power for multiple transmissions in a subframe | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-141045 | 1360 | - | Correction to test configuration of Maximum input for CA | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-141047 | 1271 | - | eICIC RF: Corrections to TDD PHICH test | 12.0.0 | 12.1.0 |
| 2014-06 | RAN#64 | R5-142069 | 1362 | - | LTE Type A performance requirements - adding TC 8.2.1.4.3 into annex F and G | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142071 | 1363 | - | Addition of connection diagram references to LTE Type A performance tests | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142072 | 1364 | - | Addition of connection diagram references to LTE Type A CSI tests | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142111 | 1365 | - | Addition of CA 3A-28A to 36.521-1 Chapter 6 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142112 | 1366 | - | Addition of CA 3A-28A to 36.521-1 Chapter 7 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142172 | 1367 | - | Addition of TC 6.2.2A.1 for CA_39C | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142176 | 1368 | - | Corrections to configured transmitted power for CA | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142177 | 1369 | - | Corrections to OCNG pattern | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142295 | 1370 | - | Updates of 6.2.5 Delta TIB for CA_3A-26A and CA_3A-27A | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142296 | 1371 | - | Updates of 7.3.3 Delta RIB for CA_3A-26A and CA_3A-27A | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142307 | 1372 | - | RF: Corrections to spurious emission requirements with NS different than NS_01 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142309 | 1373 | - | RF: Editorial corrections to the annexes | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142312 | 1374 | - | CA RF: Several corrections to some Tx tests | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142317 | 1375 | - | eICIC RF: Corrections to RI performance tests | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142343 | 1376 | - | Addition of LTE CA_39A-41A into Table 6.2.5.3-2 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142366 | 1377 | - | Update of TC 8.7.4.1 TDD sustained data rate performance for EPDCCH scheduling | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142372 | 1378 | - | Update of TC 8.8.2.1.2 FDD EPDCCH localized transmission with TM10 quasi co-location Type-B | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142376 | 1379 | - | Update of TC 8.8.2.2.2 TDD EPDCCH localized transmission with TM10 quasi co-location Type-B | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142384 | 1380 | - | Update of TC 9.3.6.1_F FDD CQI Reporting under fading conditions multiple CSI processes for CoMP | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142387 | 1381 | - | Update overview of DL reference measurement channels for EPDCCH | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142391 | 1382 | - | Uncertainties and Test Tolerances for Enhanced Performance Requirement Type A 9.3.5.1.1 and 9.3.5.1.2 CQI Test cases | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142393 | 1383 | - | Test Tolerances for eICIC 9.2.x CQI Test cases | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142394 | 1384 | - | Correction of the test cases 8.2.1.2.2 and 8.2.1.2.2_1 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142420 | 1386 | - | Corrections to Annex G.5.4 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142444 | 1387 | - | Introduction of transmission mode 10 in precoder update granularity in FDD and TDD common test parameters | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142446 | 1388 | - | Introduction of new TC 8.3.2.4.2_F TDD PDSCH Performance with DCI format 2D, non Quasi Co-located Antenna Ports, Same Cell ID and multiple NZP CSI-RS resources for CoMP | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142449 | 1389 | - | Introduction of TC 8.3.2.4.3_F TDD PDSCH Performance with DCI format 2D, non Quasi Co-located Antenna Ports, Different Cell ID, Colliding CRS and single NZP CSI-RS for CoMP | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142515 | 1390 | - | Corrections to MBMS performance test cases | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142576 | 1391 | - | Update UE Transmit OFF power for CA _39C (intra-band contiguous DL CA and UL CA) | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142588 | 1392 | - | Remove the square bracket in TC 8.3.1.3.2_F and TC 8.3.1.3.3_F | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142590 | 1393 | - | Uncertainties and Test Tolerances for eICIC PDCCH/PCFICH and PHICH demodulation tests | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142596 | 1394 | - | Addition of message exception for PHICH duration | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142597 | 1515 | - | Correction to eICIC CSI tests | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142601 | 1395 | - | Correction to title in feICIC PDSCH demodulation tests | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142605 | 1396 | - | Correction to measurement bandwidth in 6.6.2.1A.1 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142616 | 1399 | - | Correction to MBMS performance | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142672 | 1400 | - | Correction to in-band blocking requirements | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142676 | 1401 | - | Correction to 6.6.2.1A test environment | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142692 | 1402 | - | Correction to 8.2.2.1.1_1 and 8.2.2.2.1_1 applicability | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142697 | 1403 | - | 36.521-1: Corrections to section 7.3A | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142701 | 1404 | - | Introduction of NC CA TC 7.7A.4 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142703 | 1405 | - | Uncertainties and Derivation of Test Requirements for TCs 7.6.xA.4 and 7.7A.4 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142710 | 1406 | - | 36-521-1: Clarification on Averaged EVM for PRACH | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142749 | 1407 | - | Correction to CA band combo CA_3A-5A | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142750 | 1408 | - | Verification of exceptions of REFSENS requirements for carrier aggregation | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142771 | 1409 | - | Addition of CA_2A-4A and CA_5A-7A to 36.521-1 Chapter 7 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143003 | 1410 | - | 36.521-1: Editorial correction on 6.6.3.3 Initial conditions for NS_19 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143004 | 1411 | - | 36.521-1: Editorial correction on 6.6.2.2B test requirements for NS_06 or NS_07 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143005 | 1412 | - | Correction for CA_4A-12A uplink configuration in inter-band CA reference sensitivity | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143006 | 1413 | - | CA RF: Clarification of test parameters and requirements for Inter-band CA Rx tests | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143007 | 1414 | - | Corrections to SNR test for TM9 minimum requirements | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143008 | 1415 | - | Correction to ACK NAK Reporting mode for TDD CA test cases | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143009 | 1416 | - | Addition of TC 8.2.1.1.1_2 and 8.2.2.1.1_2 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143010 | 1417 | - | Correction of FDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA test cases of TCs 8.2.1.3.1_A.2 and 8.2.1.3.1_A.2_1 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143011 | 1418 | - | Updates to 8.2.1.3 FDD PDSCH Open Loop Spatial Multiplexing 2x2 related test cases | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143012 | 1419 | - | Updates to 8.2.2.3 TDD PDSCH Open Loop Spatial Multiplexing 2x2 related test cases | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143013 | 1420 | - | 36.521-1: Editorial correction on 8.2.1.4.2_A.2 message contents | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143014 | 1421 | - | Clarification the minimum conformance requirement and test parameter for 8.2.1.3.3_C1 and 8.2.1.3.3_C2 test cases | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143015 | 1422 | - | Correction to minimum test time for eICIC tests | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143025 | 1423 | - | LTE Type A performance requirements - Introduction of the new test case 8.2.1.4.3 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143026 | 1424 | - | Test Tolerances for eICIC 9.5.x RI Test cases | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143027 | 1425 | - | Uncertainties and Test Tolerances for EPDCCH 8.8.1.x demodulation Test cases | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143031 | 1426 | - | Update to operating bands for CA | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143032 | 1427 | - | Update to channel bandwidths for CA | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143033 | 1428 | - | Update to TC 6.3.2A.1 for CA_39C | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143034 | 1429 | - | Updates of 7.3A.3 Refsens for CA_3A-26A and CA_3A-27A | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143035 | 1430 | - | Addition of CA band Combo CA_2A-13A | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143036 | 1431 | - | Addition of CA_39A-41A to Refsens in chap.7 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143037 | 1432 | - | Updates to Chap.5 for CA_27B | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143038 | 1433 | - | Addition of CA_2A-4A and CA_5A-7A to 36.521-1 Clause 6 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143039 | 1434 | - | TC 8.2.1.3.3_E.1 FDD PDSCH Open Loop Spatial Multiplexing 2x2 for feICIC (non-MBSFN ABS) | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143040 | 1435 | - | TC 8.2.2.3.3_E.1 TDD PDSCH Open Loop Spatial Multiplexing 2x2 for feICIC (non-MBSFN ABS) | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143041 | 1436 | - | Corrections applicability statement including UE category for feICIC | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143042 | 1437 | - | Additions to TC 8.2.1.2.3_E.1 FDD PDSCH Transmit diversity 2x2 for feICIC (non-MBFSN ABS) | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143043 | 1438 | - | Additions to TC 8.2.2.2.3_E.1 TDD PDSCH Transmit diversity 2x2 for feICIC (non-MBSFN ABS) | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143044 | 1439 | - | Additions to TC 8.2.1.4.1_E.1 FDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x2 for feICIC (non-MBSFN ABS) | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143045 | 1440 | - | Additions to TC 8.2.2.4.1_E.1 TDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x2 for feICIC (non-MBSFN ABS) | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143046 | 1441 | - | Additions to TC 9.2.1.5_E.1 FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for feICIC (non-MBSFN ABS) | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143047 | 1442 | - | Additions to TC 9.2.1.6_E.1 TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for feICIC (non-MBSFN ABS) | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143048 | 1443 | - | Additions to TC 9.3.1.3.1_E.1 FDD CQI Reporting under fading conditions - PUSCH 3-0 for feICIC (non-MBSFN ABS) | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143049 | 1444 | - | Additions to TC 9.3.1.3.2_E.1 TDD CQI Reporting under fading conditions - PUSCH 3-0 for feICIC (non-MBSFN ABS) | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143050 | 1445 | - | Additions to TC 9.5.4.1_E.1 FDD RI Reporting - PUCCH 1-0 for feICIC (non-MBSFN ABS) | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143051 | 1446 | - | Additions to TC 9.5.4.2_E.1 TDD RI Reporting - PUCCH 1-0 for feICIC (non-MBSFN ABS) | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143052 | 1447 | - | Additions to Reporting of Channel State Information feICIC Test Cases to Annex F | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143076 | 1448 | - | Update of TC 9.3.6.2_F TDD CQI Reporting under fading conditions multiple CSI processes for CoMP | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143077 | 1449 | - | Correction parameter and reference table number in test case 8.3.2.4.1_F | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143079 | 1450 | - | Update of TC 8.7.3.1 FDD sustained data rate performance for EPDCCH scheduling | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143080 | 1451 | - | Addition of TC 8.8.3.1 FDD Localized transmission with TM10 Type B quasi co-location type | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143081 | 1452 | - | Addition of TC 8.8.3.2 TDD Localized transmission with TM10 Type B quasi co-location type | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143082 | 1453 | - | Update initial condition and test requirement in test case 8.7.1.1_A.2_1 for two 15MHz CCs | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143097 | 1454 | - | Addition of message content exception in eICIC performance tests with MBSFN ABS | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143098 | 1455 | - | Additions to TC 8.4.1.2.3_E.1 FDD PCFICH/PDCCH Transmit Diversity 2x2 for feICIC (non-MBSFN ABS) | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143099 | 1456 | - | Additions to TC 8.4.2.2.3_E.1 TDD PCFICH/PDCCH Transmit Diversity 2x2 for feICIC (non-MBSFN ABS) | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143100 | 1457 | - | Additions to TC 8.4.2.2.3_E.2 TDD PCFICH/PDCCH Transmit Diversity 2x2 for feICIC (MBSFN ABS) | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143101 | 1458 | - | Additions to Demodulation feICIC Test Cases to Annex F | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143104 | 1459 | - | Introduction of TC 8.2.2.4.2_A.3 TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4 x 2 for CA | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143105 | 1460 | - | Updates to Refsens for Intra-band non-contiguous CA | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143106 | 1461 | - | Correction of interference rejection test cases TC 8.2.2.4.3 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143107 | 1462 | - | Correction to eICIC PDSCH demodulation tests | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143113 | 1463 | - | Introduction of TC 8.2.2.1.1_A.3 TDD PDSCH Single Antenna Port Performance for CA | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143114 | 1464 | - | Additions to TC 8.4.1.2.3_E.2 FDD PCFICH/PDCCH Transmit Diversity 2x2 for feICIC (MBSFN ABS) | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143115 | 1465 | - | Update of TC 8.8.1.1 FDD distributed EPDCCH performance | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143116 | 1466 | - | Update of TC 8.8.2.1 FDD localized transmission with TM9 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143117 | 1467 | - | Update of TC 8.8.2.2 TDD localized transmission with TM9 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143118 | 1468 | - | Update sections of A.3.9 and A.3.10 reference channel for EPDCCH test | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143129 | 1469 | - | Addition of FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (intra band non-contiguous DL CA) TC 9.6.1.1_A.3 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143130 | 1470 | - | Addition of TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (intra band non-contiguous DL CA) TC 9.6.1.2_A.3 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143131 | 1471 | - | Addition of TC 6.2.4A.1 for CA_39C | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143133 | 1472 | - | Update additional spurious emissions for CA_38C (intra-band contiguous DL CA and UL CA) | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143134 | 1473 | - | Update In-band blocking for CA_39C (intra-band contiguous DL CA and UL CA) | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143135 | 1474 | - | Update Out-of-band blocking for CA_39C (intra-band contiguous DL CA and UL CA) | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143136 | 1475 | - | Update Reference sensitivity level for CA_39C (intra-band contiguous DL CA and UL CA) | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143137 | 1476 | - | RF: Corrections to CSI RMCs | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143138 | 1477 | - | CA RF: Corrections to CQI performance tests | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143140 | 1478 | - | CA RF: Corrections to performance test with power imbalance (Class C) | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143141 | 1479 | - | Correction of test case 8.2.1.1.1_1 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143142 | 1480 | - | Correction of the test applicability for test cases 8.2.1.1.1_A.2, 8.2.1.3.1_A.1, 8.2.1.3.1_A.2 and 8.2.1.4.2_A.2 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143143 | 1481 | - | Correction of the test applicability of the test cases 8.2.1.1.1_A.1, 8.2.1.4.2_A.1, 8.2.2.1.1_A.1 and 8.2.2.4.2_A.1 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143144 | 1482 | - | Correction of the applicability for the test case 8.7.1.1 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143146 | 1483 | - | Addition of TC 9.5.5.1_F.2 FDD RI Reporting with Multiple CSI processes for CoMP | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143147 | 1484 | - | Addition of TC 9.5.5.2_F.2 TDD RI Reporting with Multiple CSI processes for CoMP | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143148 | 1485 | - | Clarification of test parameters for eICIC PDCCH performance tests | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143149 | 1486 | - | Correction to Intra-band CA uplink configuration for reference sensitivity | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143151 | 1487 | - | Correction to CQI reference measurement channels | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143154 | 1488 | - | Introduction of NC CA TC 7.6.1A.4 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143155 | 1489 | - | Introduction of NC CA TC 7.6.2A.4 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143156 | 1490 | - | Introduction of NC CA TC 7.6.3A.4 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143161 | 1491 | - | Update Spurious emission band UE co-existence for CA_39C (intra-band contiguous DL CA and UL CA) | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143162 | 1492 | - | Update additional spurious emissions for CA_39C (intra-band contiguous DL CA and UL CA) | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143163 | 1493 | - | Updates to TC 7.3A.2 Reference sensitivity level for CA_27B | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143164 | 1494 | - | Updates to TC 7.4A.2 maximum input level for intra-band contiguous CA for CA_27B | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143165 | 1495 | - | Updates to TC 7.5A.2 ACS for CA_27B | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143166 | 1496 | - | Updates to 7.6.1A.2 IBB for CA of CA_27B | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143167 | 1497 | - | Updates to TC 7.6.2A.2 OOB for CA of CA_27B | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143168 | 1498 | - | Updates to TC 7.6.3A.2 Narrow band blocking for CA_27B | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143169 | 1499 | - | Updates to TC 7.7A.2 Spurious response for CA_27B | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143170 | 1500 | - | Updates to TC 7.8.1A.2 Wideband intermodulation for CA_27B | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143175 | 1502 | - | Updates to 8.7.2.1 TDD sustained data rate performance related test cases | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143178 | 1504 | - | Additions to TC 9.5.5.1_F.1 FDD RI Reporting with Single CSI process | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143179 | 1505 | - | Additions to TC 9.5.5.2_F.1 TDD RI Reporting with Single CSI process | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143181 | 1506 | - | Splitting of FDD CA TM3 and soft buffer management tests | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143182 | 1507 | - | Removal of soft buffer test points from FDD TM3 test cases in 8.2.1.3.1 and move to new soft buffer test cases in section 8.2.1.3.1A | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143183 | 1508 | - | Splitting of TDD CA TM3 and soft buffer management tests | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143184 | 1509 | - | Removal of soft buffer test points from TDD TM3 test cases in 8.2.2.3.1 and move to new soft buffer test case in section 8.2.2.3.1A | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143188 | 1512 | - | Addition of TC 8.2.2.7 TDD Carrier Aggregation with power imbalance | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143189 | 1513 | - | Corrections to test case 8.2.1.4.2_1, 8.2.2.4.1_1, and 8.2.2.4.2_1 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143190 | 1514 | - | Uncertainties and Test Tolerances for CoMP Test case 8.3.1.3.2_F | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143203 | 1385 | - | Throughput calculation for eICIC demodulation requirements | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143204 | 1503 | - | Correction to eICIC TDD RI requirements | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143205 | 1510 | - | Updates to 8.7.1.1_A FDD sustained data rate performance for CA related test cases | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143206 | 1511 | - | Updates to 8.7.2.1_A TDD sustained data rate performance for CA related test cases | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143207 | 1397 | - | Correction to CA Rx test cases | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143208 | 1398 | - | Correction to call setup in 8.7.1 and 8.7.2 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143216 | 1501 | - | Updates to 8.7.1.1 FDD sustained data rate performance related test cases | 12.1.0 | 12.2.0 |
| 2014-09 | RAN#65 | R5-144068 | 1516 | - | Uncertainties and test tolerances for test case 8.2.2.2.4 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144072 | 1517 | - | Uncertainties and test tolerances for test case 8.3.1.1.3 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144074 | 1518 | - | Uncertainties and test tolerances for test case 8.3.2.1.4 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144086 | 1519 | - | Corrections to TC 8.2.2.1.1_A.3 TDD PDSCH Single Antenna Port Performance for CA | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144087 | 1520 | - | Corrections to TC 8.2.2.4.2_A.3 TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4 x 2 for CA | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144088 | 1521 | - | Test Tolerances to Annex F for TC 8.2.2.1.1_A.3 and TC 8.2.2.4.2_A.3 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144093 | 1522 | - | Test Tolerances for TC 9.5.5.1_F.1 FDD RI Reporting with Single CSI process | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144094 | 1523 | - | Test Tolerances for TC 9.5.5.2_F.1 TDD RI Reporting with Single CSI process | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144095 | 1524 | - | Addition of OCNG pattern for TM10 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144096 | 1525 | - | Uncertainties and Test Tolerances for TC 9.5.5.1_F.1+9.5.5.2_F.1 in Annex F | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144098 | 1526 | - | Test Tolerances for TC 8.2.1.4.1_E.1 FDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x2 for feICIC (non-MBSFN ABS) | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144099 | 1527 | - | Test Tolerances for TC 8.2.2.4.1_E.1 TDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x2 for feICIC (non-MBSFN ABS) | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144101 | 1528 | - | Test Tolerances for TC 8.4.1.2.3_E.1 FDD PCFICH/PDCCH Transmit Diversity 2x2 for feICIC (non-MBSFN ABS) | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144102 | 1529 | - | Test Tolerances for TC 8.4.2.2.3_E.1 TDD PCFICH/PDCCH Transmit Diversity 2x2 for feICIC (non-MBSFN ABS) | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144104 | 1530 | - | Test Tolerances for TC 8.4.1.2.3_E.2 FDD PCFICH/PDCCH Transmit Diversity 2x2 for feICIC (MBSFN ABS) | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144105 | 1531 | - | Test Tolerances for TC 8.4.2.2.3_E.2 TDD PCFICH/PDCCH Transmit Diversity 2x2 for feICIC (MBSFN ABS) | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144107 | 1532 | - | Uncertainties and Test Tolerances to Annex F for Demodulation feICIC | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144111 | 1533 | - | Test Tolerances for TC 9.2.1.5_E.1 FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for feICIC (non-MBSFN ABS) | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144112 | 1534 | - | Test Tolerances for TC 9.2.1.6_E.1 TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for feICIC (non-MBSFN ABS) | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144114 | 1535 | - | Test Tolerances for TC 9.3.1.3.1_E.1 FDD CQI Reporting under fading conditions - PUSCH 3-0 for feICIC (non-MBSFN ABS) | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144115 | 1536 | - | Test Tolerances for TC 9.3.1.3.2_E.1 TDD CQI Reporting under fading conditions - PUSCH 3-0 for feICIC (non-MBSFN ABS) | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144117 | 1537 | - | Test Tolerances for TC 9.5.4.1_E.1 FDD RI Reporting - PUCCH 1-0 for feICIC (non-MBSFN ABS) | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144118 | 1538 | - | Test Tolerances for TC 9.5.4.2_E.1 TDD RI Reporting - PUCCH 1-0 for feICIC (non-MBSFN ABS) | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144120 | 1539 | - | Uncertainties and Test Tolerances to Annex F for Reporting of Channel State Information feICIC | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144122 | 1540 | - | Message contents updates for CSI feICIC test cases | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144123 | 1541 | - | Corrections to Annex G for feICIC | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144173 | 1542 | - | RF: Corrections to spurious emission band co-existence test | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144174 | 1543 | - | RF: Missing test coverage for Band 31 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144176 | 1544 | - | IR RF: Correction to TCs 8.3.1.1.3 and 8.3.2.1.4 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144244 | 1545 | - | Correction of clause 6.2.4.5 in 36.521-1 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144248 | 1546 | - | Addition of operating band 30 to TS 36.521-1, clause 5 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144283 | 1547 | - | Add static propagation condition matrix for 1 x 2 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144299 | 1548 | - | Update of TC 8.3.1.3.2_F CoMP | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144300 | 1549 | - | Update of TC 8.3.1.3.3_F CoMP | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144301 | 1550 | - | Update of TC 8.3.2.4.2_F CoMP | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144302 | 1551 | - | Update of TC 8.3.2.4.3_F CoMP | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144310 | 1552 | - | Update minimum requirements and test tolerance for TC 8.7.3.1 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144311 | 1553 | - | Update minimum requirements and test tolerance for TC 8.7.4.1 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144312 | 1554 | - | Update minimum requirements and test tolerance for TC 8.8.2.1 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144313 | 1555 | - | Update minimum requirements and test tolerance for TC 8.8.2.2 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144347 | 1556 | - | Correction to Inter-band CA operating bands | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144352 | 1557 | - | Correction to eICIC performance test cases | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144353 | 1558 | - | Correction to UL/DL configuration in CQI requirements | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144393 | 1559 | - | RF: Corrections to A-MPR test requirements | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144416 | 1560 | - | Update configurations of CA_39C in A-MPR for CA | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144420 | 1561 | - | Update configurations of CA_39C in Additional spurious emissions for CA | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144436 | 1562 | - | Update of test applicabilities for TDD PDSCH Transmit Diversity | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144439 | 1563 | - | Update of test applicability for TDD PDSCH Transmit Diversity 4x2 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144441 | 1564 | - | Update test applicabilities for TDD interference rejection test cases | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144474 | 1565 | - | Updates to TC 8.7.2.1 and 8.7.2.1_1 for TDD SDR | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144766 | 1567 | - | Correction to CA Rx Blocking TCs | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144775 | 1568 | - | Correction of the ePDCCH TCs for applying default parameter | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144787 | 1566 | - | Addition of new TC 8.2.1.3.1_A.3 FDD PDSCH Open Loop Spatial Multiplexing 2x2 for intra-band NC DL CA | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144801 | 1569 | - | Removal of TBDs in Clause 6.2.4A.1 (A-MPR for UL intra-band cont. CA), CA_NS_02 and 03, correction of Test Requirements | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144802 | 1570 | - | Correction to in-band blocking for CA requirements | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144803 | 1571 | - | Corrections to TM9 SNR requirements for eDL-MIMO | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144804 | 1572 | - | Addition of exception message for TDD MBMS | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144805 | 1573 | - | RF: Corrections to sustained data rate tests | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144806 | 1574 | - | Correction of TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (intra band contiguous DL CA) TC 9.6.1.2_A.1 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144807 | 1575 | - | Correction of TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (intra band non-contiguous DL CA) TC 9.6.1.2_A.3 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144808 | 1576 | - | Correction to the test cases 8.3.2.1.1, 8.3.2.1.2, 8.3.2.1.3 and 8.3.2.2.1 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144809 | 1577 | - | Correction to RF General section on CA configurations | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144814 | 1578 | - | Update TC 9.3.6.1_F FDD CQI Reporting under fading conditions multiple CSI processes for CoMP | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144815 | 1579 | - | Update TC 9.3.6.2_F TDD CQI Reporting under fading conditions multiple CSI processes for CoMP | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144817 | 1580 | - | Addition of CoMP AWGN CQI Tests to Test Requirements Derivation in Annex F | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144818 | 1581 | - | Update minimum requirements and test tolerance for TC 8.8.3.1 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144819 | 1582 | - | Update minimum requirements and test tolerance for TC 8.8.3.2 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144820 | 1583 | - | Uncertainties and test tolerances for test cases 8.2.1.3.3_E.1 and 8.2.2.3.3_E.1 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144821 | 1584 | - | Uncertainties and Test Tolerances for feICIC Test cases 8.2.1.2.3_E.1 and 8.2.2.2.3_E.1. | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144822 | 1585 | - | Uncertainties and test tolerances for test case 8.2.2.4.3 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144824 | 1586 | - | Uncertainties and test tolerances for test cases 9.3.5.2.1 and 9.3.5.2.2 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144826 | 1587 | - | eICIC Test Tolerance references to TS 36.904 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144827 | 1588 | - | Addition of CA aQI Tests to Test Requirements Derivation in Annex F | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144828 | 1589 | - | ePDCCH RF: Corrections to sustained data rate tests | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144836 | 1590 | - | Update to CoMP test case 9.5.5.1_F.2 and 9.5.5.2_F.2 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144839 | 1591 | - | New CA band combination CA_NC_42 and CA_4-27-Update to 36.521-1 Chapter 6 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144840 | 1592 | - | New CA band combination CA_NC_42 and CA_4-27-Update to 36.521-1 Chapter 7 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144841 | 1593 | - | Addition of CA band Combo CA_7A-28A | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144842 | 1594 | - | Addition of CA_2A-2A to 36.521-1 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144844 | 1595 | - | Existing CA band combination CA_3A-28A update to 36.521-1 7.3 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144845 | 1596 | - | Addition of new CA band combo CA_2A-5A to 36.521-1 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144859 | 1597 | - | New TC: TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (inter band DL CA) | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144873 | 1599 | - | Correction of clause 6.3.4.1.4 in 36.521-1 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144874 | 1600 | - | Removal of transition period for Band 28 test frequency definition | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144875 | 1601 | - | Clarification of tables used in TDD Intra band non-contiguous DL CA TCs | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144876 | 1602 | - | Introduction of new Wideband Intermodulation test for Intra-band NC CA | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144879 | 1603 | - | New CA band combination CA_NC_42 and CA_4-27-Update to 36.521-1 Chapter 5 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144880 | 1604 | - | Update reference sensitivity level for CA (intra-band contiguous DL CA without UL CA) | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144881 | 1605 | - | Update In-band blocking for CA (intra-band contiguous DL CA without UL CA) | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144891 | 1606 | - | CA RF: Adopting test frequencies selection to the new structure proposed for TS 36.508 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144894 | 1607 | - | Corrections to 8.3.1.2.1_D_1 FDD PDSCH Dual-layer Spatial Multiplexing for eDL-MIMO (Release 11 and forward) | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144895 | 1608 | - | Corrections to 8.3.2.2.1_D_1 TDD PDSCH Dual-layer Spatial Multiplexing for eDL-MIMO (Release 11 and forward) | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144896 | 1609 | - | Correct errors in TDD PDSCH Single Antenna Port Performance test case | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144897 | 1610 | - | Separation of minimum requirements for CA UE PDSCH demodulation from single carrier | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144898 | 1611 | - | Introduction of FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (intra band contiguous DL CA) TC 9.6.1.1_A.1 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144899 | 1612 | - | Correction of FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (inter band DL CA) TC 9.6.1.1_A.2 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144900 | 1613 | - | Correction of FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (intra band non-contiguous DL CA) TC 9.6.1.1_A.3 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144901 | 1614 | - | Updates to 8.2.1.4.2_A.1/A.2 FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA related test cases | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144902 | 1615 | - | Updates to 8.2.2.3.1_A TDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144903 | 1616 | - | Updates to 8.2.2.4.2_A TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144904 | 1617 | - | Updates to 8.2.2.3.1 TDD PDSCH Open Loop Spatial Multiplexing 2x2 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144905 | 1618 | - | Updates to 8.2.2.3.2 TDD PDSCH Open Loop Spatial Multiplexing 4x2 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144906 | 1619 | - | Updates to 8.2.2.4 TDD PDSCH Closed Loop Single/Multi Layer Spatial Multiplexing 2x2 and 4x2 | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144907 | 1620 | - | Updates to 9.2.x CQI Reporting under AWGN conditions | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144916 | 1621 | - | Message contents updates for TC 9.5.5.1_F.1+9.5.5.2_F.1 CoMP | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144917 | 1622 | - | FDD CQI Reporting under AWGN conditions - Single CSI Process TC 9.2.4.1_F - Editor Notes Resolution | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144918 | 1623 | - | TDD CQI Reporting under AWGN conditions - Single CSI Process TC 9.2.4.2_F - Editor Notes Resolution | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144920 | 1624 | - | Addition of SDR test case 8.7.1.1_A.3 for intra-band NC DL CA | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144922 | 1625 | - | New TC: TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (inter band DL CA) | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144923 | 1626 | - | New TC: TDD PDSCH Single Antenna Port Performance for CA (inter-band DL CA) | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144924 | 1627 | - | New TC: TDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (inter-band DL CA) | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144925 | 1628 | - | New TC: TDD sustained data rate performance for CA (inter-band DL CA) | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144926 | 1629 | - | Update Out-of-band blocking for CA (intra-band contiguous DL CA without UL CA) | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144928 | 1630 | - | Message contents updates for Demod feICIC test cases | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144930 | 1631 | - | Addition of new TC 8.2.2.3.1A_A.3 TDD Soft buffer management test for intra-band NC DL CA | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144931 | 1632 | - | Addition of new TC 8.2.1.4.2_A.3 FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (intra band NC DL CA) | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144932 | 1633 | - | Updates to TCs 8.2.1.3.1_A.1/A.2 for FDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144933 | 1634 | - | Updates to 8.7.1.1_A FDD SDR for CA related test cases | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144934 | 1635 | - | Addition of new TC 8.2.2.3.1A_A.2 TDD Soft buffer management test for inter-band DL CA | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144936 | 1636 | - | Updates of Annex F.1.4 and F.3.4 for newly added performance test cases | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144938 | 1637 | - | Updates to TC 8.2.2.1 TDD PDSCH Single Antenna Port Performance (Cell-Specific Reference Symbols) related test cases | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144939 | 1638 | - | Updates to 8.2.2.2 TDD PDSCH Transmit Diversity Performance (Cell-Specific Reference Symbols) | 12.2.0 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144942 | 1640 | - | Updates to TC 9.6.1.1_A.2 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 for CA (inter band DL CA) | 12.2.0 | 12.3.0 |
| 2014-12 | RAN#66 | R5-145014 | 1641 | - | Corrections to CA Blocking TCs minimum requirements | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145015 | 1642 | - | Correction to CA Spurious Response TC minimum requirements | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145125 | 1645 | - | RF: Review of definition of CSI measurement channels with dynamic coding rate | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145128 | 1646 | - | RF: Review of references to CSI measurement channels with dynamic coding rate | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145175 | 1647 | - | New CA band combination CA_1A-3A - Updates of 5.2A Operating bands | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145176 | 1648 | - | New CA band combination CA_1A-3A - Updates of 5.4.2A Channel Bandwidth | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145177 | 1649 | - | New CA band combination CA_1A-3A - Updates of 6.2.5 Delta TIB | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145178 | 1650 | - | New CA band combination CA_1A-3A - Updates of 7.3.3 Delta RIB | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145189 | 1651 | - | eICIC-RF: Corrections to PDCCH performance tests | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145196 | 1652 | - | RF: Corrections to RMC-s tables | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145210 | 1653 | - | CA-RF: Correction to OBW requirement | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145222 | 1654 | - | Introduction of UE maximum output power for CA_42C | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145223 | 1655 | - | Introduction of Spurious emission band UE co-existence for CA_42C | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145229 | 1656 | - | Update the minimum requirement and test requirements of TC 8.8.3 localized transmission with TM10 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145232 | 1657 | - | Existing CA band combination CA_4A-27A and new CA band combination CA_41A-42A update to 36.521-1 section 5.4.2A.1 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145234 | 1658 | - | New CA band combination CA_41A-42A update to 36.521-1 section 5.2A | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145235 | 1659 | - | New CA band combination CA_41A-42A update to 36.521-1 section 6.2.5 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145236 | 1660 | - | New CA band combination CA_41A-42A update to 36.521-1 section 7.3.3 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145291 | 1661 | - | CQI reporting in AWGN and in fading: CQI indices in set | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145311 | 1662 | - | Clarification for CA ACS and Wideband Intermodulation testcases | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145374 | 1663 | - | New CA band combination CA_18A-28A - Updates of 5.2A Operating bands | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145375 | 1664 | - | New CA band combination CA_18A-28A - Updates of 5.4.2A Channel bandwidth | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145376 | 1665 | - | New CA band combination CA_18A-28A - Updates of 6.2.5.3 Delta TIB | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145377 | 1666 | - | New CA band combination CA_18A-28A - Updates of 7.3A Delta RIB | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145407 | 1667 | - | Alignment with 36.521-2 on UE categories for COMP | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145425 | 1668 | - | Removal of Non-contiguous Allocation within a Component Carrier Testpoints from UL CA MPR, Spectrum Emissions and ACLR Tests | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145431 | 1669 | - | Correction on parameters for high speed train scenario | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145436 | 1670 | - | Update TC 9.3.6 CQI Reporting under fading conditions Multiple CSI processes for CoMP | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145437 | 1671 | - | New CA band combination 1+11 and 8+11 û Introduction of 1+11 and 8+11 to 36.521-1 chapter 5 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145438 | 1672 | - | New CA band combination 1+11 and 8+11 û Introduction of 1+11 and 8+11 to 36.521-1 chapter 6 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145439 | 1673 | - | New CA band combination 1+11 and 8+11 û Introduction of 1+11 and 8+11 to 36.521-1 chapter 7 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145465 | 1674 | - | Correction to TC 8.2.1.1.1_A.1 FDD PDSCH Single Antenna Port Performance for CA | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145479 | 1675 | - | Corrections to throughput calculation for feICIC demodulation minimum requirements | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145480 | 1676 | - | Test Tolerances for TC 8.5.1.2.3_E.1 FDD PHICH Transmit Diversity 2x2 for feICIC (non-MBSFN ABS) | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145481 | 1677 | - | Test Tolerances for TC 8.5.2.2.3_E.1 TDD PHICH Transmit Diversity 2x2 for feICIC (non-MBSFN ABS) | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145486 | 1678 | - | Uncertainties and Test Tolerances to Annex F for PHICH feICIC | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145487 | 1679 | - | Corrections to Annex G for minimum test time for demodulation for feICIC | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145505 | 1680 | - | Updates to TDD SDR CA test cases | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145532 | 1681 | - | Addition of missing bandwidth combination sets for inter-band CA | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145536 | 1682 | - | Additions to CoMP Demod test cases 8.3.1.3.1_F and 8.3.2.4.1_F | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145539 | 1683 | - | Addition of Band 30 in Max output power test cases | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145540 | 1684 | - | Addition of Band 30 in Max power reduction test cases | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145543 | 1685 | - | Addition of Band 30 in Spurious emission UE co-existence test cases | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145559 | 1686 | - | Correction to 6.3.5A.1.1 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145638 | 1687 | - | Correction to HPUE Applicability statements in TS 36.521-1 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145800 | 1688 | - | RF: Modification of test description to refer to State 2A-RF | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145803 | 1689 | - | Removal of Non-contiguous Allocation within a Component Carrier Testpoints from UL CA A-MPR and Additional Spectrum/Spurious Emissions Tests | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145804 | 1690 | - | Addition of RF requirements in later releases | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145805 | 1691 | - | eDL-MIMO RF: Correction to TC 8.3.1.1.1_D | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145808 | 1692 | - | Correction on support of a bandwidth combination set | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145809 | 1693 | - | Correction to TC 8.7.1.1_A FDD sustained data rate performance for CA | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145810 | 1694 | - | Correction to TC 8.2.2.1.1_A.1 TDD PDSCH Single Antenna Port Performance for CA | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145811 | 1695 | - | Corrections to TC 8.2.2.1.1_A.3 TDD PDSCH Single Antenna Port Performance for CA | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145812 | 1696 | - | Corrections to TC 8.2.2.4.2_A.3 TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4 x 2 for CA | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145813 | 1697 | - | Corrections to Annex G for CA Enhancements | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145825 | 1698 | - | Uncertainties and TT for CoMP test case 9.5.5.1_F.2 and 9.5.5.2_F.2 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145835 | 1699 | - | Update Uncertainties for RI Test cases 9.5.x | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145839 | 1700 | - | Update of Demodulation of PDSCH CoMP TCs | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145840 | 1701 | - | Uncertainties and Test Tolerances for CoMP Test cases 8.3.1.3.3_F, 8.3.2.4.2_F, 8.3.2.4.3_F | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145841 | 1702 | - | Correction to CoMP TDD RI test case | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145846 | 1703 | - | Introduction of CA_42C in section 5 of TS 36.521-1 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145847 | 1704 | - | New CA band combination CA_4A-27A and CA_41A-42A update to 36.521-1 section 7.3A.3 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145848 | 1705 | - | Introduction of receiver requirements for CA_42C into TS 36.521-1 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145851 | 1706 | - | Addition of operating band 30 to TS 36.521-1 Clause 7 for receiver tests | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145853 | 1707 | - | Sections 3 and 4 update on 3DL CA WI | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145854 | 1708 | - | Addition of new 2DL(CA_1A-42A and CA_19A-42A) and 3DL CA band combinations(CA_1A-3A-19A, CA_1A-19A-21A, CA_1A-42C and CA_19A-42C) | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145855 | 1709 | - | New CA band combination CA_18A-28A - Updates of 7.3A Refsens | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145856 | 1710 | - | Update of CA Maximum input level Test cases | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145857 | 1711 | - | Update of CA ACS Test cases | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145858 | 1712 | - | Update of CA Blocking and Spurious Response Test cases | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145859 | 1713 | - | Update of CA Rx Intermodulation Test cases | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145872 | 1714 | - | Update to applicability for TM9 test cases | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145874 | 1715 | - | RF: Correction to A-MPR requirements with NS_20 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145876 | 1716 | - | Applicability of CSI requirements for CA capability | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145880 | 1717 | - | Addition of Band 30 in Additional Max power reduction test cases | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145881 | 1718 | - | Addition of Band 30 in Additional SEM test cases | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145882 | 1719 | - | Addition of Band 30 in Additional Spurious emission test cases | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145883 | 1720 | - | Update the minimum requirement and test requirements of TC 8.8.2 localized transmission with TM9 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145884 | 1721 | - | New CA band combination CA_1A-3A - Updates of 7.3A Refsens | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145885 | 1722 | - | New CA band combination CA_NC_41A and CA_NC_42A update to 36.521-1 section 7.4A.4 and 7.5A.4 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145886 | 1723 | - | New CA band combination CA_NC_41A and CA_NC_42A update to 36.521-1 section 7.6.1A.4, 7.6.2A.4 and 7.6.3A.4 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145887 | 1724 | - | New CA band combination CA_NC_41A and CA_NC_42A update to 36.521-1 section 7.8.1A.4 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145888 | 1725 | - | Existing CA band combination CA_39C: update bandwidth combination to CQI test | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145889 | 1726 | - | Existing CA band combination CA_39C: update bandwidth combination to PDSCH TM1 test | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145890 | 1727 | - | Existing CA band combination CA_39C: update bandwidth combination to PDSCH TM3 test | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145891 | 1728 | - | Existing CA band combination CA_39C: update bandwidth combination to PDSCH TM4 test | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145893 | 1729 | - | 7A-7A NC CA RX RF Tests Testpoint Correction | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145896 | 1730 | - | CA-RF: Correction to ACLR test points | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145898 | 1731 | - | Intraband Contiguous CA RF Tests Testpoint Choice Clarification | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145899 | 1732 | - | Updates to Spurious emission band UE co-existence test cases 6.6.3.2 and 6.6.3.2A.1 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145900 | 1733 | - | Chapter 7 RX RF Tests Intra-band contiguous Test Configuration Tables Cleanup | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145901 | 1734 | - | Updates to FDD and TDD PDSCH Soft buffer management test (inter-band DL CA) | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145902 | 1735 | - | Updates to TM3 CA demodulation tests for release independence | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145917 | 1736 | - | Addition of CRS assistance data in message contents for feICIC demodulation test cases | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145922 | 1737 | - | Introduction of TC 8.2.1.1.1_A.3 FDD PDSCH Single Antenna Port Performance for CA | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145926 | 1739 | - | New CA band combination CA_NC_41A and CA_NC_42A update to 36.521-1 section 7.3A.4 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145933 | 1740 | - | Existing CA band combination CA_39C: update annex tables for 20+15MHz | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145934 | 1741 | - | Reconstruction of FDD SDR test cases based on Release independent rule | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145935 | 1742 | - | RF: Cleanup of sustained data rate test cases | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145936 | 1743 | - | Applicability of Demodulation requirements for CA capability | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145975 | 1643 | - | CA-RF: Review of test frequencies clause 8 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145976 | 1644 | - | CA-RF: Review of test frequencies clause 9 | 12.3.0 | 12.4.0 |
| 2015-03 | RAN#67 | R5-150112 | 1744 | - | Addition of CA_2-30 to Clause 5.2A of TS 36.521-1. | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150135 | 1745 | - | Addition of CA_5-30 to Clause 5.2A of TS 36.521-1. | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150136 | 1746 | - | Addition of CA_5A-30A to Clause 5.4.2A of TS 36.521-1. | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150169 | 1747 | - | Addition of CA_4A-5A-30A to Clause 5.4.2A of TS 36.521-1. | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150181 | 1748 | - | Addition of new TC 9.3.6.1_F.2 FDD CQI Reporting under fading conditions with Three CSI processes for CoMP | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150184 | 1749 | - | Addition of new TC 9.3.6.2_F.2 TDD CQI Reporting under fading conditions with Three CSI processes for CoMP | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150186 | 1840 | - | Addition of TT for new TCs 9.3.6.1_F.1, 9.3.6.1_F.2, 9.3.6.1_F.3, 9.3.6.2_F.1, 9.3.6.2_F.2 and 9.3.6.2_F.3 | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150209 | 1750 | - | Corrections to 1 PRB allocation performance in presence of MBSF | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150210 | 1751 | - | Corrections to the Common Test Parameters for User-specific Reference Symbols | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150214 | 1752 | - | Test Tolerances for 8.3.1.2.1_D_1 FDD PDSCH Dual-layer Spatial Multiplexing for eDL-MIMO Rel-11 | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150215 | 1753 | - | Test Tolerances for 8.3.2.2.1_D_1 TDD PDSCH Dual-layer Spatial Multiplexing for eDL-MIMO Rel-11 | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150216 | 1754 | - | Uncertainties and Test Tolerances to Annex F for PDSCH for eDL-MIMO Rel-11 | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150234 | 1755 | - | CA RF: Corrections to power imbalance test | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150252 | 1756 | - | Update of non-contiguous CA Rx Test Cases | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150364 | 1757 | - | ePDCCH RF: Corrections to sustained data rate test | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150373 | 1758 | - | Introduction of new CA band combinations 5+25 and 12+25 for Transmitter Characteristics | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150409 | 1759 | - | Correction in cl.3 of 36.521-1 | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150415 | 1760 | - | Correction to message exceptions for eICIC performance test cases | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150416 | 1761 | - | Clean up on Performance and CSI test cases for eICIC and feICIC | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150417 | 1762 | - | Correction to test procedure in CQI Reporting under AWGN for feICIC | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150420 | 1763 | - | Correction to minimum test time for feICIC performance tests in G.3A.5 | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150437 | 1764 | - | New CA band combination CA_1A-28A - Updates of 5.2A Operating bands | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150438 | 1765 | - | New CA band combination CA_1A-28A - Updates of 5.4.2A Channel bandwidth | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150440 | 1766 | - | New CA band combination CA_1A-28A - Updates of 6.2.5.3 Delta TIB | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150441 | 1767 | - | New CA band combination CA_1A-28A - Updates of 7.3.3 Delta RIB | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150452 | 1768 | - | New CA band combination CA_1A-18A-28A - Updates of 5.4.2A Channel bandwidth | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150453 | 1769 | - | New CA band combinations (CA_1A-41A, CA_1A-41C) - Updates of 5.2A Operating bands | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150455 | 1770 | - | New CA band combinations (CA_26A-41A, CA_26A-41C) - Updates of 5.2A Operating bands | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150456 | 1771 | - | New CA band combination CA_1A-18A-28A - Updates of 5.2A Operating bands | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150460 | 1772 | - | Correction to the description of duplex mode related to CA_18A-28A | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150461 | 1773 | - | Correction to the test frequencies and test CC combinations for CA_18A-28A | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150472 | 1774 | - | Correction to requirements for test case 7.6.2A.1 and 7.6.2A.2 | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150476 | 1775 | - | Removal of Note 1 in requirements of TC 6.2.2A | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150478 | 1776 | - | Addition of &#916:RIB,c to requirements of blocking and spurious response test cases | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150479 | 1777 | - | Correction to PUSCH configuration details for 36.521-1 4x2 CA test cases | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150505 | 1778 | - | Added TT in annex for TDD inter band CA performance tests | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150506 | 1779 | - | Updated TT in TDD inter band CA performance tests | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150522 | 1780 | - | Addition of CA_1A-20A to TS 36.521-1 | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150543 | 1781 | - | Addition of 2A-12A and 5A-13A 2DL Interband CA to 36.521-1 | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150606 | 1782 | - | Clarification on bandwidth to be tested | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150800 | 1783 | - | Correction to 6.6.3.2 of Tx Spurious | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150801 | 1784 | - | Correction in 6.2.5A and 6.5.1A of 36.521-1 | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150802 | 1785 | - | Fixing numbering in Table 6.6.2.3A.1.4.1-1 | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150803 | 1786 | - | Change of test points for A-MPR in band 26 | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150809 | 1787 | - | Addition of BCS1 to 25A-25A CA | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150810 | 1788 | - | Introduction of CA_1A-7A to TS 36.521-1 | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150811 | 1789 | - | Correction to test points for intra-band contiguous receiver test cases | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150816 | 1790 | - | Addition of CA_2A-30A to Clause 7.3.3 of TS 36.521-1. | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150817 | 1791 | - | Addition of CA_2A-30A to Clause 6.2.5 of TS 36.521-1. | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150818 | 1792 | - | Addition of CA_2A-30A to Clause 5.4.2A of TS 36.521-1. | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150819 | 1793 | - | Addition of CA_4A-30A to Clause 7.3.3 of TS 36.521-1. | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150820 | 1794 | - | Addition of CA_4A-30A to Clause 6.2.5 of TS 36.521-1. | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150821 | 1795 | - | Addition of CA_4-30 to Clause 5.2A of TS 36.521-1. | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150822 | 1796 | - | Addition of CA_4A-30A to Clause 5.4.2A of TS 36.521-1. | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150823 | 1797 | - | Addition of CA_5A-30A to Clause 7.3.3 of TS 36.521-1. | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150824 | 1798 | - | Addition of CA_5A-30A to Clause 6.2.5 of TS 36.521-1. | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150825 | 1799 | - | Addition of CA_4-5-30 to Clause 5.2A of TS 36.521-1. | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150826 | 1800 | - | Addition of new TC FDD PDSCH Single Antenna Port Performance for CA (3DL CA) | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150827 | 1801 | - | Addition of CA_2A-2A-13A to TS 36.521-1 | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150828 | 1802 | - | Addition of 40D,41D,41C-41A and 25A-25A BCS1 to Channel BW for CA tables | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150829 | 1803 | - | Addition of multiple interband 3DL CA Combinations to Channel BW for CA tables | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150848 | 1804 | - | Addition of new TC 9.3.6.1_F.1 FDD CQI Reporting under fading conditions with Single CSI process for CoMP | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150849 | 1805 | - | Addition of new TC 9.3.6.1_F.3 FDD CQI Reporting under fading conditions with Four CSI processes for CoMP | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150850 | 1806 | - | Addition of new TC 9.3.6.2_F.1 TDD CQI Reporting under fading conditions with Single CSI process for CoMP | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150851 | 1807 | - | Addition of new TC 9.3.6.2_F.3 TDD CQI Reporting under fading conditions with Four CSI processes for CoMP | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150860 | 1808 | - | Introduction of new CA band combinations 5+25 and 12+25 for Receiver Characteristics | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150861 | 1809 | - | Addition of CA_1A-7A-20A to TS 36.521-1 | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150862 | 1810 | - | Update of minimum requirements for TC 6.2.5 Configured UE transmitted Output Power | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150863 | 1811 | - | Updates of minimum requirement to reference sensitivity for CA | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150865 | 1812 | - | Addition of new TC TDD PDSCH Single Antenna Port Performance for CA (3DL CA) | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150866 | 1813 | - | Addition of new TC FDD Sustained data rate performance for CA (3DL CA) | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150867 | 1814 | - | Addition of new TC TDD Sustained data rate performance for CA (3DL CA) | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150869 | 1839 | - | Addition of Cell IDs for 3DL CA Rx tests | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150870 | 1815 | - | New CA band combinations (CA_1A-41A, CA_1A-41C) - Updates of 5.4.2A Channel bandwidth | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150871 | 1816 | - | New CA band combinations (CA_26A-41A, CA_26A-41C) - Updates of 5.4.2A Channel bandwidth | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150935 | 1817 | - | Change in UE co-existence 6.6.3.2 harmonic emissions exceptions | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150874 | 1818 | - | CA RF: Updates to in-band emissions requirements | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150875 | 1819 | - | Correction to test procedure of TC 7.6.1A.4 | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150880 | 1820 | - | Update of CA sustained data rate tests | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150881 | 1821 | - | Aligning Bandwidth Combination Sets with latest core specification | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150892 | 1822 | - | Update to TC 7.5A.4 Adjacent Channel Selectivity (ACS) for CA (intra-band non-contiguous DL CA without UL CA) | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150897 | 1823 | - | New CA band combination CA_1A-28A - Updates of 7.3A Refsens | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150928 | 1824 | - | Updates to Spurious emission band UE co-existence test case 6.6.3.2 | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150906 | 1825 | - | Updates to FDD and TDD RI Reporting test cases for CoMP | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150929 | 1827 | - | Addition of CA capability in Rx tests | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150913 | 1828 | - | Addition of Multi-Cluster PUSCH with One Uplink Carrier test cases to 36.521-1 | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150915 | 1829 | - | Addition of CA capability and applicability changes to Section 8 36.521-1 FDD TM3 CA test cases | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150916 | 1830 | - | Changes to Annex F of 36.521-1 related to CA capability and applicability changes to FDD CA test cases | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150917 | 1831 | - | Correction to CA applicability and test rules | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150918 | 1832 | - | Addition of CA capability and applicability changes to 36.521-1 FDD TM1 CA test cases | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150919 | 1833 | - | Update to LTE CA Sustained Data rate test cases | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150920 | 1834 | - | CA demod Soft Buffer test case variants merge | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150921 | 1835 | - | Addition of CA capability and applicability changes to Section 8 36.521-1 FDD TM4 CA test cases | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150922 | 1836 | - | Addition of CA capability and applicability changes to FDD chap9 CA test cases | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150925 | 1837 | - | Introduction of Test Case for Maximum input level for 3DL CA | 12.4.0 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150926 | 1838 | - | Addition of new TC of reference sensitivity for 3DL CA | 12.4.0 | 12.5.0 |
| 2015-06 | RAN#68 | R5-151067 | 1841 | - | Removal of Extreme Conditions from TC 6.6.2.1A test environment | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151114 | 1843 | - | RF: Missing power class information | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151116 | 1845 | - | CA RF: Clarification of PHICH configuration | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151117 | 1846 | - | RF: Corrections to CSI RMCs references | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151121 | 1850 | - | ePDCCH RF: Corrections to sustained data rate test | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151122 | 1851 | - | feICIC RF: Updates to TC 9.2.1.6_E.1 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151123 | 1852 | - | feICIC RF: Corrections to CSI TCs | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151151 | 1853 | - | Correction to the requirements of test case 6.6.2.1A | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151152 | 1854 | - | Addition of Rx parameter for test case 7.6.2A.1 and 7.6.2A.2 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151181 | 1855 | - | CA RF: Corrections to the applicability clause | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151195 | 1857 | - | Corrections to CSI Reference Measurement Channel | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151213 | 1871 | - | Updating Annex A relevant for the introduced of eDL MIMO enhancement Tests | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151214 | 1872 | - | Updating Annex B Beam steering approach for eDL MIMO enhancement Tests | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151215 | 1873 | - | Updating Annex I relevant for the introduced of eDL MIMO enhancement Tests | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151231 | 1874 | - | CoMP TCs applicability update section 8 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151237 | 1876 | - | Update of CoMP TCs 9.2.4.1_F and 9.2.4.2_F | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151238 | 1877 | - | Update of CoMP TCs in sections 9.3.6.1_F and 9.3.6.2_F | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151256 | 1884 | - | Update of CoMP TCs in section 8 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151257 | 1885 | - | Update of CoMP TCs 9.5.5.1_F.1 and 9.5.5.2_F.1 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151329 | 1902 | - | Addition of band 32 to clause 6 in 36.521-1 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151366 | 1912 | - | Removal of double table 7.6.1B.5-2 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151367 | 1913 | - | Corrections to UL CA Test Cases | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151369 | 1915 | - | Editorial corrections to TC 6.6.1 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151425 | 1940 | - | Addition of Band 32 in Receiver test cases | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151487 | 1955 | - | Introduction of new test case 8.9.1.1.1 MTC | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151515 | 1962 | - | Correction to test case 6.3.3A.1: UE Transmit OFF power for CA (intra-band contiguous DL CA and UL CA) | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151561 | 1963 | - | Update to TX test case 6.6.3.3 Additional spurious emissions | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151591 | 1976 | - | 36.521-1: CA Soft buffering corrections | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151616 | 1978 | - | Introduction of new test case 9.7.1.1 MTC | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151639 | 1980 | - | Test points correction in test case 7.3A.2 for CA_23B in 36.521-1 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151802 | 1875 | 1 | CoMP TCs applicability update section 9 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151805 | 1904 | 1 | Clarification to spurious emission requirement for the edge of spurious domain | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151806 | 1842 | 1 | Missing NS Value for Band 30 Reference Sensitivity Testing and Missing Message Content Exceptions | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151807 | 1891 | 1 | Correction of UL RB Allocation in Interband DL CA Testcases | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151808 | 1898 | 1 | Correction of CA_25A-25A wrong allocation in Chapter 7 Table 7.3A.4.4.1-1 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151809 | 1914 | 1 | Updates for interferer placement in non-contiguous CA | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151811 | 1903 | 1 | Correction of UEs category for TDD sustained data rate performance | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151812 | 1952 | 1 | Updates to 36.521-1 Annex F and G | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151813 | 1916 | 1 | Clean up of PDSCH performance test cases | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151814 | 1924 | 1 | Clean up of eDL-MIMO test cases | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151815 | 1925 | 1 | Clean up of TM9 interference model test cases | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151816 | 1926 | 1 | Correction to eICIC performance test cases | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151817 | 1927 | 1 | Correction to CQI and RI reporting test in feICIC | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151818 | 1928 | 1 | Correction to PDSCH demodulation test in feICIC | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151819 | 1929 | 1 | Clean up on PCFICH/PDCCH demodulation test in feICIC | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151820 | 1930 | 1 | Clean up on PHICH demodulation test in feICIC | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151821 | 1932 | 1 | Correction of PMI test cases in the section 9.4.1 and 9.4.2 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151822 | 1966 | 1 | Removal of the correlation matrix of HST propagation conditions | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151825 | 1979 | 1 | Updates to CoMP test cases 9.5.5.x | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151854 | 1909 | 1 | Addition of new TC FDD PDSCH 2x2 Open Loop Performance for 3DL CA | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151855 | 1910 | 1 | Addition of new TC FDD PDSCH 4x2 Closed Loop Performance for 3DL CA | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151856 | 1967 | 1 | Introduction of TC TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4X2 for CA (3DL CA) | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151857 | 1968 | 1 | Introduction of TC TDD PDSCH Open Loop Spatial Multiplexing 2X2 for CA (3DL CA) | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151858 | 1969 | 1 | New CA band combination CA_1A-41A and CA_26A-41A - Updates of 6.2.5.3 Delta TIB | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151859 | 1970 | 1 | New CA band combination CA_1A-41A and CA_26A-41A - Updates of 7.3.3 Delta RIB | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151884 | 1878 | 1 | New TCs: FDD PMI Reporting with 4Tx enhanced codebook - PUSCH 1-2 (Multiple PMI) for eDL-MIMO Enhancement | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151885 | 1879 | 1 | New TCs: TDD PMI Reporting with 4Tx enhanced codebook - PUSCH 1-2 (Multiple PMI) for eDL-MIMO Enhancement | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151891 | 1901 | 1 | Addition of band 32 to clause 5 in 36.521-1 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151894 | 1941 | 1 | Addition of test case 6.2.2E - UE Maximum Output Power for UE category 0 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151895 | 1943 | 1 | Introduction of UE category 0 in section 4 of 36.521-1 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151896 | 1954 | 1 | Update of SNR definition in clause 8.1.1 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151897 | 1859 | 1 | Introduction of TC 6.3.5A.1.2 Power Control Absolute Power Tolerance for CA | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151899 | 1860 | 1 | Introduction of TC 6.3.5A.1.3 Power Control Absolute Power Tolerance for CA | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151900 | 1883 | 1 | New CA 2UL Tx test case 6.2.2A.3 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151901 | 1861 | 1 | Introduction of TC 6.3.5A.2.3 Power Control Relative Power Tolerance for CA | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151902 | 1862 | 1 | Introduction of TC 6.3.5A.2.2 Power Control Relative Power Tolerance for CA | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151903 | 1863 | 1 | Introduction of TC 6.7A.2 Transmit intermodulation for CA | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151921 | 1886 | 1 | Updates of minimum requirement for reference sensitivity for CA | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151922 | 1887 | 1 | Addition of test points and test requirements for 3DL inter-band CA | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151926 | 1977 | 1 | Update of 7.6.1A.1 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151928 | 1918 | 1 | Correction to CQI reference measurement channels | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151929 | 1849 | 1 | CA RF: Corrections to sustained data rate test | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151930 | 1946 | 1 | Update two band and three band information in test case 7.3 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151931 | 1888 | 1 | Update of Maximum input level for CA (3DL CA without UL CA) | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151932 | 1950 | 1 | Introduction of TDD FDD CA with TDD as Pcell for PDSCH Single Antenna Port Performance | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151933 | 1951 | 1 | Introduction of TDD FDD CA with FDD as Pcell for PDSCH Single Antenna Port Performance | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151934 | 1956 | 1 | Introduction to test case 9.6.1.1_A.2: FDD CQI Reporting under AWGN conditions – PUCCH 1-0 (3DL CA) | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151935 | 1957 | 1 | Introduction to test case 9.6.1.2_A.2: TDD CQI Reporting under AWGN conditions – PUCCH 1-0 (3DL CA) | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151936 | 1965 | 1 | Addition of CA capability and applicability for 3 DL CCs in Chapter 8 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151937 | 1971 | 1 | New CA band combination CA_1A-41A and CA_26A-41A - Updates of 7.3A Refsens | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151938 | 1972 | 1 | New CA band combination CA_1A-18A-28A - Updates of 7.3.3 Delta RIB and 7.3A Refsens | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151940 | 1958 | 1 | Introduction to test case 6.3.3A.3: UE Transmit OFF power for CA (intra-band non-contiguous DL CA and UL CA) | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151941 | 1959 | 1 | Introduction to test case 6.3.4A.1.3: General ON/OFF time mask for CA (intra-band non-contiguous DL CA and UL CA) | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151942 | 1960 | 1 | Introduction to test case 6.6.1A.3: UE Occupied bandwidth for CA (intra-band non-contiguous DL CA and UL CA) | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151943 | 1961 | 1 | Introduction to test case 6.6.2.1A.3: Spectrum Emission Mask for CA (intra-band non-contiguous DL CA and UL CA) | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151944 | 1893 | 1 | Merge of TDD CQI Reporting under AWGN conditions – PUCCH 1-0 for CA cases | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151945 | 1936 | 1 | Introduction of TC 7.6.2A.5 Out-of-band blocking for CA (3DL CA without UL CA) | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151946 | 1983 | 1 | Introduction of TC 7.6.3A.5 Narrow-band blocking for CA (3DL CA without UL CA) | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151947 | 1937 | 1 | Introduction of TC 7.7A.5 Spurious response for CA (3DL CA without UL CA) | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151948 | 1935 | 1 | Introduction of TC 7.6.1A.5 In-band blocking for CA (3DL CA without UL CA) | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151950 | 1844 | 1 | CA RF: PCC/SCC mapping onto physical CC-s definition cleanup | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-152000 | 1848 | 1 | CA RF: Corrections to power imbalance test | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-152001 | 1953 | 1 | Correction to 9.3.1.3.1_E.1 FDD CQI Reporting test | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-152002 | 1858 | 1 | Corrections to Reference Measurement Channel | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-152003 | 1911 | 1 | Update of DL RMCs for PDSCH performance requirements | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-152004 | 1947 | 1 | Addition of CA capability, applicability changes and core spec updates to Section 8 36.521-1 TDD TM1 CA test cases | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-152005 | 1948 | 1 | Addition of CA capability, applicability changes and core spec updates to Section 8 36.521-1 TDD TM3 CA test cases | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-152006 | 1949 | 1 | Addition of CA capability, applicability changes and core spec updates to Section 8 36.521-1 TDD TM4 CA test cases | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-152007 | 1974 | 1 | Editorial correction to 8.2.1.1.1_A.1 and 8.2.1.3.1_A.1 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-152013 | 1856 | 1 | Addition of Chapter 7 3DL CA Test Configuration Tables | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-152017 | 1938 | 1 | Introduction of test case 7.8.1A.5 Wideband intermodulation for CA (3DL CA without UL CA) | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-152018 | 1942 | 1 | Addition of test case 7.3E - Reference sensitivity level for UE category 0 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-152020 | 1882 | 1 | New CA 2UL Tx test case 6.2.2A.2 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-152021 | 1944 | 1 | Introduction of test case Maximum Power Reduction (MPR) for CA (inter-band DL CA and UL CA) | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-152022 | 1945 | 1 | Introduction of test case Maximum Power Reduction (MPR) for CA (intra-band non-contiguous DL CA and UL CA) | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-152025 | 1922 | 1 | Corrections to spurious emission band co-existence limit tables in clause 6 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-152027 | 1896 | 1 | Update Reference Channel and Minimum test time for TDD CA power imbalance demodulation test | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-152028 | 1897 | 1 | Update TDD CA BW combination to power imbalance test | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-152031 | 1908 | 1 | Update of TC8.2.1.1.1_A.2:FDD PDSCH Single Antenna Performance for 3DL CA | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-152032 | 1900 | 1 | Addition of Multiple 3DL CA Combinations to Chapter 7 Delta RIB and REFSENS Exceptions Tables | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-152033 | 1906 | 1 | Clarification of the power of interferer with deltaRIBc | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-152038 | 1889 | 1 | Introduction of ACS Test Case for 3DL CA without UL CA | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-152043 | 1923 | 2 | Addition and alignment of all Rel-12 CA configurations in Section 5 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-152133 | 1939 | 2 | NS value for intra-band contiguous CA configurations not allowed A-MPR | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-152136 | 1981 | 2 | 36.521-1: DL Sustained Data Rate clarifications | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | - | - | - | section 8.2.1.4.3 which was deleted by mistake due to a wrong implementation of R5-150921 was restored | 12.5.0 | 12.6.0 |
| 2015-09 | RAN#69 | R5-153100 | 1991 | - | Corrections to EVM and Carrier leakage CA test cases | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153101 | 1992 | - | Update of EVM minimum requirements | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153243 | 2017 | - | CA RF: Minor corrections | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153244 | 2018 | - | eDL-MIMO RF: Minor corrections | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153245 | 2019 | - | CA RF: Correction to applicability of SDR test | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153247 | 2021 | - | eICIC and feICIC RF: Corrections to PDSCH RMCs used for PDCCH performance | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153300 | 2029 | - | Addition of new TC of Frequency error for inter-band CA | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153305 | 2033 | - | Addition of new TC of Reference sensitivity level for 2UL inter-band CA | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153317 | 2037 | - | CA RF: Addition of missing requirements for CA_4A-12A | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153318 | 2038 | - | RF: Clarifications for TC 6.6.3.2 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153322 | 2042 | - | RF: Correction to TC 6.6.3.3A | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153362 | 2053 | - | Clean up for Tx test cases | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153364 | 2055 | - | Correction to DL parameters in CA Rx tests | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153365 | 2056 | - | Correction to 7.6.1A.4 In-band blocking for non-contiguous CA | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153367 | 2058 | - | Removable of PCFICH_RA in 8.4 and 8.5 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153371 | 2062 | - | Correction to CQI reporting under AWGN for eDL-MIMO | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153372 | 2063 | - | Correction to CQI reporting under fading(PUSCH3-1) for eDL-MIMO | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153373 | 2064 | - | Correction to CQI reporting under fading(PUCCH1-1) for eDL-MIMO | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153375 | 2066 | - | Correction to RI reporting test for eDL-MIMO | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153377 | 2068 | - | Correction to CQI reporting test for TM9 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153378 | 2069 | - | Correction to feICIC CQI reporting test procedure in 9.2.1.5_E.1 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153423 | 2075 | - | Updates of ?TIB,c values to TC 6.2.5 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153460 | 2091 | - | Correction of 1A-3A-5A CA REFSENS DL Allocation | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153462 | 2092 | - | Addition of several definitions to chapter 3 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153474 | 2095 | - | Applicability Update of Intra-Band DL-only CA testcases in Chapter 7 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153613 | 2122 | - | Introduction of Receiver Spurious emissions test case for Carrier Aggregation in DL-only bands | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153624 | 2126 | - | New CA 2UL Tx test case 6.6.2.3A.2 Adjacent Channel Leakage power Ratio for CA (inter-band DL CA and UL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153629 | 2127 | - | Test Tolerances for test case 6.3.4A.1.3 General ON/OFF time mask for CA (intra-band contiguous DL CA and UL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153630 | 2128 | - | Test Tolerances for test case 6.3.4A.1.1 General ON/OFF time mask for CA (intra-band contiguous DL CA and UL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153633 | 2130 | - | Test Tolerances for test case 6.6.1A.3 Occupied bandwidth for CA (intra-band non-contiguous DL CA and UL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153634 | 2131 | - | Test Tolerances for test case 6.6.2.1A.3 Spectrum emission mask for CA (intra-band non-contiguous DL CA and UL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153646 | 2135 | - | 36.521-1_Reference corrections in test 6.6.3.2 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153671 | 2145 | - | Resubmission: CA RF: Clarification of PHICH configuration | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153800 | 2070 | 1 | Correction to applicability of feICIC demod test cases | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153801 | 2057 | 1 | Correction to test procedure in 7.8.1A.4 Wideband intermodulation for non-contiguous CA | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153802 | 2090 | 1 | Correction to Testpoint Selection in Chapter 7 NC CA Testcases | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153803 | 2048 | 1 | Correction to title of table A 3.2-3 and A 3.2-4 in annex A.3.2 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153804 | 2061 | 1 | Correction to message exceptions in performance test(User-Specific RS) for eDL-MIMO | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153805 | 2065 | 1 | Correction to PMI reporting test for eDL-MIMO | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153806 | 2067 | 1 | Correction to TM9 interference model test cases | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153807 | 2078 | 1 | Updates to 8.7.2.1.A_1 TDD Sustained data rate performance for CA (2DL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153810 | 2087 | 1 | Correction to test point selection table for all chap8/9 FDD and TDD 2DLCA cases | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153821 | 2012 | 1 | Update to FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (3DL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153822 | 2024 | 1 | Addition of annex F for Reference sensitivity for 3DL CA | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153823 | 2039 | 1 | CA RF: Update of test points for CA_12B | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153827 | 2094 | 1 | Addition of 3DL CA default XB-YA configuration to REFSENS test configuration table | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153836 | 1999 | 1 | Introduction of SUMIMO for LTE Type C performance TC 8.2.2.3.1B | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153837 | 2111 | 1 | Addition of Reference measurement channel and overview reference for multi-cell test in Annex | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153838 | 2112 | 1 | Introduction of FDD TM3 test case for LTE Type C performance requirements (8.2.1.3.1B) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153839 | 2113 | 1 | Introduction of FDD TM3 multi-cell test case with TM1 interference for LTE Type C performance requirements (8.2.1.3.1C) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153840 | 2114 | 1 | Introduction of TDD TM3 multi-cell test case with TM1 interference for LTE Type C performance requirements (8.2.2.3.1C) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153841 | 2115 | 1 | Introduction of FDD TM4 test case for LTE Type C performance requirements (8.2.1.4.2A) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153842 | 2116 | 1 | Introduction of TDD TM4 test case for LTE Type C performance requirements (8.2.2.4.2A) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153843 | 2117 | 1 | Introduction of TDD TM8 test case for LTE Type C performance requirements (8.3.2.2.2) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153844 | 2118 | 1 | Introduction of FDD TM9 test case for LTE Type C performance requirements (8.3.1.2.2) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153845 | 2119 | 1 | Addition of Enhanced performance requirements Type C definition | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153848 | 1997 | 1 | Update of eDL MIMO enhancement TC 9.4.2.3.3_G | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153849 | 1998 | 1 | Update of eDL MIMO enhancement TC 9.4.2.3.4_G | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153851 | 2000 | 1 | New TC: 9.7.1.2 TDD CQI reporting definition under AWGN conditions for UE category 0 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153852 | 2125 | 1 | RMC definitions for UE category 0 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153853 | 2123 | 1 | Additions to test case 6.2.2E - UE Maximum Output Power for UE category 0 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153854 | 2124 | 1 | Changes to test case 7.3E - Reference sensitivity level for UE category 0 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153855 | 2026 | 1 | Addition of new TC of Transmit off power for inter-band CA | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153856 | 2034 | 1 | Addition of annex F for 2UL CA test cases | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153858 | 2045 | 1 | Introduction to test case 6.2.4A.3: Additional Maximum Power Reduction (A-MPR) for CA (intra-band non-contiguous DL CA and UL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153859 | 2088 | 1 | EVM for Intra-band contiguous UL CA for non-equal Channel BWs | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153867 | 2150 | - | Adding missing subclauses in Annex A.2 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153868 | 2008 | 1 | Align Test Procedure and Test requirements for 7.5A.5 ACS 3DL CA | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153869 | 2010 | 1 | Align Test requirements for 7.4A.5 Max Input 3DL CA | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153870 | 2132 | 1 | Completion to Test cases 9.6.1.1_A.2 and 9.6.1.2_A.2: FDD/TDD CQI Reporting under AWGN conditions – PUCCH 1-0 (3DL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153873 | 2035 | 1 | Updates of minimum requirements of Spurious emission band UE co-existence | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153874 | 2001 | 1 | New CA band combination CA_1A-5A-7A - Updates of 7.3.3 Delta RIB and 7.3A Refsens Test Configuration | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153879 | 2036 | 1 | CA RF: Update of intraband contiguous blocking requirements | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153880 | 2096 | 1 | Update of Reference sensitivity for CA_39A-41C and CA_39C-41A | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153881 | 2121 | 1 | Updates to test case 7.8.1A.5 Wideband intermodulation for CA (3DL CA without UL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153882 | 2148 | - | New CA band combinations (CA_1A-41C, CA_26A_41C) - Updates of 7.3.3 Delta RIB | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153884 | 2147 | - | Addition of Multiple Inter-band CA Configurations to 7.3A.4 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153885 | 2011 | 1 | Update to FDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (3DL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153886 | 2013 | 1 | Update to TDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (3DL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153887 | 2014 | 1 | Update to TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (3DL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153888 | 2076 | 1 | Updates to 8.2.2.1.1_A.2 TDD PDSCH Single Antenna Port Performance for CA (3DL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153889 | 2077 | 1 | Updates to 8.7.1.1.A_2 FDD Sustained data rate performance for CA (3DL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153890 | 2079 | 1 | Updates to 8.7.2.1.A_2 TDD Sustained data rate performance for CA (3DL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153891 | 2086 | 1 | Updates to TDD FDD CA test cases for PDSCH Single Antenna Port Performance | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153892 | 2105 | 1 | Update of FDD PDSCH Single Antenna Port Performance for CA (3DL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153893 | 2106 | 1 | TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for TDD PCell (2DL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153894 | 2107 | 1 | TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for TDD PCell (3DL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153895 | 2149 | 1 | New CA band combinations (CA_1A-41C, CA_26A-41C) - Updates of 7.3A Refsens | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153902 | 1993 | 1 | Introduction of eDL MIMO enhancement TC 9.3.7.1_G | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153903 | 1994 | 1 | Introduction of eDL MIMO enhancement TC 9.3.7.2_G | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153904 | 1995 | 1 | Introduction of eDL MIMO enhancement TC 9.4.1.4.1_G | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153905 | 1996 | 1 | Introduction of eDL MIMO enhancement TC 9.4.1.4.2_G | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153906 | 2016 | 1 | UE Cat 0 RF: Addition to Annex I | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153907 | 2097 | 1 | Addition of test case 7.4E - Maximum input level for UE category 0 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153908 | 2120 | 1 | Additions to test case 6.2.3E - Maximum Power Reduction (MPR) for UE category 0 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153909 | 2144 | 1 | Additions to test case 6.2.4E – Additional Maximum Power Reduction (A-MPR) for UE category 0 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153912 | 2027 | 1 | Addition of new TC of ON/OFF time mask for inter-band CA | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153913 | 2028 | 1 | Addition of new TC of Occupied bandwidth for inter-band CA | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153914 | 2030 | 1 | Addition of new TC of Spectrum emission mask for inter-band CA | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153915 | 2031 | 1 | Addition of new TC of Transmitter Spurious emissions for inter-band CA | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153916 | 2032 | 1 | Addition of new TC of Spurious emission band UE co-existence for inter-band CA | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153917 | 2046 | 1 | Introduction to test case 6.2.5A.2: Configured UE transmitted Output Power for CA (inter-band DL CA and UL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153918 | 2047 | 1 | Introduction to test case 6.2.5A.3: Configured UE transmitted Output Power for CA (intra-band non-contiguous DL CA and UL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153920 | 2007 | 1 | Update of 6.2.3_2 MPR for Multi-Cluster PUSCH | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153921 | 2054 | 1 | Correction to lower tolerances for CA_NS_05 in A-MPR | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153922 | 2129 | 1 | Corrections to test case 6.6.1A.1 Occupied bandwidth for CA (intra-band contiguous DL CA and UL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153923 | 2043 | 1 | CA RF: Clean Up of UL resource allocation for interband 2DL CA | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153924 | 2002 | 1 | Correction of minimum requirements and test procedures for 8.2.2.1.1_A.1 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153925 | 2003 | 1 | Corrections to 8.2.2.3.1_A.1 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153926 | 2059 | 1 | Correction to the title in 2DL CA performance test cases | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153927 | 2060 | 1 | Correction to FDD soft buffer test for CA | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153928 | 2134 | 1 | 36.521-1_CA power imbalance test updates | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153944 | 2022 | 2 | Updates of minimum requirements of reference sensitivity for 3DL CA | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153945 | 2023 | 2 | Updates of Reference sensitivity for 3DL CA | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153947 | 2146 | 1 | UE Category Corrections to test case 8.2.1.4.2_A.1 from 36.521-1 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153949 | 1985 | 1 | Correction to RC.2 TDD Nr. HARQ Proc. | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-154002 | 1986 | 1 | Introduction of TC 9.6.1.3.1 TDD FDD CA CQI Reporting – PUCCH 1-0 for FDD PCell (2DL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-154003 | 1987 | 1 | Introduction of TC 9.6.1.3.2 TDD FDD CA CQI Reporting – PUCCH 1-0 for FDD PCell (3DL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-154004 | 1988 | 1 | Introduction of TC 9.6.1.4.1 TDD FDD CA CQI Reporting – PUCCH 1-0 for TDD PCell (2DL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-154005 | 1989 | 1 | Introduction of TC 9.6.1.4.2 TDD FDD CA CQI Reporting – PUCCH 1-0 for TDD PCell (3DL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-154006 | 2098 | 1 | Addition of test case 7.5E - Adjacent Channel Selectivity (ACS) for UE category 0 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-154007 | 2099 | 1 | Addition of test case 7.6.1E - In-band blocking for UE category 0 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-154008 | 2100 | 1 | Addition of test case 7.6.2E - Out-of-band blocking for UE category 0 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-154009 | 2101 | 1 | Addition of test case 7.6.3E - Narrow band blocking for UE category 0 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-154010 | 2102 | 1 | Addition of test case 7.7E - Spurious response for UE category 0 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-154011 | 2103 | 1 | Addition of test case 7.8.1E - Wide band Intermodulation for UE category 0 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-154012 | 2104 | 1 | Addition of test case 7.9E - Spurious emissions for UE category 0 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-154014 | 2136 | 1 | 36.521-1_Test requirements correction in test 6.3.5.2 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-154015 | 2080 | 1 | Addition of 8.7.5.1.1 TDD FDD CA Sustained data rate performance for FDD PCell (2DL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-154016 | 2081 | 1 | Addition of 8.7.5.1.2 TDD FDD CA Sustained data rate performance for FDD PCell (3DL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-154017 | 2082 | 1 | Addition of 8.7.5.2.1 TDD FDD CA Sustained data rate performance for TDD PCell (2DL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-154018 | 2083 | 1 | Addition of 8.7.5.2.2 TDD FDD CA Sustained data rate performance for TDD PCell (3DL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-154019 | 2049 | 1 | Update to TC 7.6.1A.5 In-band blocking for CA (3DL CA without UL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-154022 | 2050 | 1 | Update to TC 7.6.2A.5 Out-of-band blocking for CA (3DL CA without UL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-154023 | 2051 | 1 | Update to TC 7.6.3A.5 Narrow band blocking for CA (3DL CA without UL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-154024 | 2052 | 1 | Update to TC 7.7A.5 Spurious response for CA (3DL CA without UL CA) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-154026 | 2084 | 2 | Addition of SCE-L1 clause numbering principle in Annex I | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-154030 | 2009 | 1 | Revise Power Control Relative power tolerance for 2 UL CA test case 6.3.5A.2.1 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-154031 | 2020 | 1 | RF: Applicability of CSI requirements to UE Category 1 (for 36.521-1) | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | - | - | - | update of the "non-specific references" in section 2 according to the approved R5-153582 and an action point on ETSI MCC | 12.6.0 | 12.7.0 |
| 2015-12 | RAN#70 | R5-155112 | 2156 | - | Addition of simplified CA testing method (Annex C.3.5) into 36.521-1 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155113 | 2157 | - | Uncertainty and Test Tolerance for FDD TM3 test case for LTE Type C performance requirements (8.2.1.3.1B) | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155116 | 2160 | - | Uncertainty and Test Tolerance for TDD TM8 test case for LTE Type C performance requirements (8.3.2.2.2) | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155117 | 2161 | - | Uncertainty and Test Tolerance for FDD TM9 test case for LTE Type C performance requirements (8.3.1.2.2) | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155118 | 2162 | - | Uncertainty and Test Tolerance for FDD TM3 multi-cell test case with TM1 interference for LTE Type C performance requirements (8.2.1.3.1C) | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155119 | 2163 | - | Uncertainty and Test Tolerance for TDD TM3 multi-cell test case with TM1 interference for LTE Type C performance requirements (8.2.2.3.1C) | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155205 | 2169 | - | Addition of test time for testing throughput ratios for eDL MIMO Enhancement | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155300 | 2206 | - | Update of editor's note for TC 6.6.2.2A.1 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155309 | 2208 | - | ProSe: Update of general chapters in 36.521-1 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155312 | 2211 | - | ProSe: Update of annexes in 36.521-1 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155313 | 2212 | - | Update of UE categories for R8 in 36.521-1 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155316 | 2214 | - | Update of UE categories for R11 in 36.521-1 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155360 | 2215 | - | Test description for test case 6.3.3A.3: UE Transmit OFF power for CA (intra-band non-contiguous DL CA and UL CA) | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155361 | 2216 | - | Correction to Test cases 9.6.1.1_A.1 and 9.6.1.2_A.1: FDD/TDD CQI Reporting under AWGN conditions – PUCCH 1-0 (2DL CA) | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155394 | 2226 | - | Updates of the test configuration about occupied bandwidth for CA (inter-band DL CA and UL CA) | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155401 | 2228 | - | Updates of message contents for ON/OFF time mask for 2UL inter-band CA | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155405 | 2232 | - | Updates of Refsens for 2UL inter-band CA | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155406 | 2233 | - | Addition of CA_3A-42A to 7.3A.3 and applicability updates for all inter-band 2DL CA receiver test cases | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155409 | 2235 | - | Updatest of CA demodulation applicabilities in general section | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155420 | 2241 | - | Changes to general clauses for Dual Connectivity | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155425 | 2246 | - | Introduction of the reference and abbreviations for Dual Connectivity | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155426 | 2247 | - | Removal of the duplicate clause 8.2.1.4.2_A.2.5 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155444 | 2253 | - | Clean up of Tx test cases | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155446 | 2255 | - | Correction to Rx UL CA test cases | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155447 | 2256 | - | Correction to reference channel for CQI requirements | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155448 | 2257 | - | Correction to UL RMC for single carrier SDR | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155450 | 2259 | - | Correction to CoMP demodulation requirements | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155452 | 2261 | - | Correction to Enhanced Performance Type A CQI reporting test | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155453 | 2262 | - | Correction to CQI and RI Reporting test cases for feICIC | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155460 | 2265 | - | CA RF: Update 3DL Refsense test requirements | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155462 | 2267 | - | Corrections to Refsense minimum requirements | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155464 | 2269 | - | CA RF: Update of test points for Rx inter-band test cases | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155471 | 2274 | - | Corrections to message contents for CSI 3DL CA TCs 9.6.1.3.2+9.6.1.4.2 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155474 | 2275 | - | Corrections to CA demodulation tests | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155476 | 2276 | - | Corrections to message exceptions for CSI-RS setting | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155478 | 2278 | - | Updates to Annexes A.2 and A.3 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155530 | 2282 | - | Addition of measurement uncertainty and test tolerances for TC 9.7.1.2 TDD CQI reporting definition under AWGN conditions for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155543 | 2283 | - | Correction to Test Parameters for the test cases 10.1 and 10.2 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155554 | 2289 | - | Correction to 6.6.3.3A.1 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155668 | 2325 | - | Addition of CA configuration CA_2A-4A-5A in Reference sensitivity level for 3DL CA | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155670 | 2326 | - | Correction to DL RMCs for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155709 | 2331 | - | Editorial corrections to TC 8.2.2.1.1_A.1 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155745 | 2346 | - | Correction to CQI test cases for LTE CA | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155801 | 2288 | 1 | Correction to 6.2.4A.1 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155802 | 2254 | 1 | Correction to 2DL CA Rx test cases | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155803 | 2266 | 1 | CA RF: Correction to message contents for 2DL Max Input Power TCs | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155804 | 2213 | 1 | Update of UE categories for R10 in 36.521-1 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155807 | 2176 | 1 | Add new LC MTC test case 9.7.2.1 FDD CQI Reporting under fading conditions for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155808 | 2177 | 1 | Add new LC MTC test case 9.7.2.2 TDD CQI Reporting under fading conditions for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155809 | 2318 | 1 | Test case 6.2.4E – Additional Maximum Power Reduction (A-MPR) for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155810 | 2321 | 1 | Additions to test case 6.3.3E – UE Transmit OFF Power for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155811 | 2322 | 1 | Additions to test case 6.3.4E.1 – General ON/OFF time mask for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155812 | 2329 | 1 | Additions to test case 6.3.5E.1 – Power Control Absolute power tolerance for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155813 | 2330 | 1 | Additions to test case 6.3.5E.2 – Power Control Relative power tolerance for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155814 | 2333 | 1 | Additions to test case 6.5.1E – Frequency Error for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155815 | 2334 | 1 | Additions to test case 6.5.2.1E.1 – Error Vector Magnitude for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155816 | 2336 | 1 | Additions to test case 6.5.2.2E – Carrier Leakage for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155817 | 2338 | 1 | Additions to test case 6.5.2.3E – In-band emissions for non allocated RB for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155818 | 2339 | 1 | Additions to test case 6.5.2.4E – EVM equalizer spectrum flatness for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155819 | 2342 | 1 | Additions to test case 6.2.5E – Configured Transmitted Power for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155840 | 2158 | 1 | Uncertainty and Test Tolerance for FDD TM4 test case for LTE Type C performance requirements (8.2.1.4.2A) | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155841 | 2159 | 1 | Uncertainty and Test Tolerance for TDD TM4 test case for LTE Type C performance requirements (8.2.2.4.2A) | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155842 | 2165 | 1 | Update of SUMIMO for LTE Type C performance TC 8.2.2.3.1B | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155844 | 2151 | 1 | New CA band combination CA_1A-3A-8A - Updates of 7.3A.5 RefSens | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155845 | 2152 | 1 | New CA band combination CA_1A-3A-8A - Updates of 7.3.3 Delta RIB | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155846 | 2183 | 1 | LTE Type B performance requirements - Addition of fixed reference channels into Annex A | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155847 | 2184 | 1 | LTE Type B performance requirements - Introduction of new interference models into Annex B.6 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155848 | 2182 | 1 | LTE Type B performance requirements - Changes to clause 3 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155850 | 2236 | 1 | Addition of new test cases for TDD FDD CA PDSCH Open Loop for FDD PCell(2DL and 3DL) | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155851 | 2224 | 1 | Update of 2DL TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for TDD PCell (8.2.3.3.2.1) | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155852 | 2225 | 1 | Update of 3DL TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for TDD PCell (8.2.3.3.2.2) | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155853 | 2248 | 1 | Update for Annex for TDD/FDD Perf test cases | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155855 | 2250 | 1 | Clean up of 3DL CA Rx test cases | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155857 | 2280 | 1 | Completion of Testcase 7.3A.5 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155859 | 2272 | 1 | CA RF: Correction to message contents for 3DL Max Input Power | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155860 | 2327 | 1 | Correction of BW to be tested and CA Capability to be tested for TM3 Test Cases | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155861 | 2328 | 1 | 36.521-1 – CA_2A-12B updates | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155892 | 2168 | 1 | Correction of test cases title for eDL MIMO Enhancement test cases | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155893 | 2263 | 1 | Corrections to eDL MIMO enhancement TC 9.3.7.1_G+9.3.7.2_G | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155894 | 2264 | 1 | Corrections to eDL MIMO enhancement TC 9.4.1.4.1_G+9.4.1.4.2_G | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156006 | 2229 | 1 | Addition of initial conditions for frequency error for 2UL inter-band CA | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156007 | 2230 | 1 | Updates of SEM for 2UL inter-band CA | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156008 | 2231 | 1 | Updates of test requirements for general spurious for 2UL inter-band CA | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156013 | 2210 | 1 | ProSe: Additions to Annexes for RF testing | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156014 | 2209 | 1 | ProSe: Reference sensitivity test | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156016 | 2242 | 1 | Introduction of FDD PDSCH Closed Loop Multi-Layer Spatial Multiplexing 4x2 for Dual Connectivity | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156017 | 2243 | 1 | Introduction of TDD PDSCH Closed Loop Multi-Layer Spatial Multiplexing 4x2 for Dual Connectivity | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156018 | 2244 | 1 | Introduction of FDD sustained data rate performance for Dual Connectivity | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156019 | 2245 | 1 | Introduction of TDD sustained data rate performance for Dual Connectivity | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156020 | 2207 | 1 | Update of RB configurations for TC 7.3A.5 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156021 | 2222 | 1 | Addition of soft-buffer test cases for FDD-TDD 2DL CA (8.2.3.2.1A, 8.2.3.2.2A) | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156022 | 2234 | 1 | Delta RIB,c updates for 3DL CA supporting UE | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156023 | 2237 | 1 | Addition of new test cases for TDD FDD CA PDSCH Closed Loop 4x2 for FDD PCell (2DL and 3DL) | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156024 | 2239 | 1 | Correction to test point selection table for TDD-FDD CA cases | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156025 | 2251 | 1 | Correction to 7.3A.5 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156026 | 2252 | 1 | Update CA perf test cases in clause 8 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156027 | 2271 | 1 | CA RF: Update to 3DL Refsense test configurations | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156028 | 2281 | 1 | Improvements to 3DL OOB Testcase (7.6.2A.5) | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156029 | 2295 | 1 | Updates to 8.7.5 about TDD-FDD CA SDR performance test | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156030 | 2337 | 1 | 36.521-1: CA_1A-3A-20A & CA_1A-7A-20A updates | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156031 | 2346 | 1 | Addition of 2x2 FDD-TDD CA test cases with TDD PCELL | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156035 | 2170 | 1 | New TC: 8.9.1.1.2 FDD Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols) for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156036 | 2171 | 1 | New TC: 8.9.1.2.2 TDD Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols) for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156037 | 2173 | 1 | Add new LC MTC test case 8.9.1.1.3 FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156038 | 2174 | 1 | Add new LC MTC test case 8.9.1.2.1 TDD PDSCH Transmit Diversity for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156039 | 2175 | 1 | Add new LC MTC test case 8.9.1.2.3 TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156040 | 2189 | 1 | Addition of Minimum test time for Demodulation performance for UE category 0 in Annex G | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156041 | 2191 | 1 | Introduction of new test case 8.9.2.2.1 MTC | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156042 | 2240 | 1 | Update of TC 9.7.1.2 TDD CQI reporting definition under AWGN conditions for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156043 | 2315 | 1 | Update test requirement for MTC test cases: 7.6.1E, 7.6.2E and 7.6.3E | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156044 | 2316 | 1 | Addition of Maximum Test System Uncertainty and Derivation of Test Requirements in Annex F for MTC test cases | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156045 | 2317 | 1 | Additions to test case 6.2.3E - Maximum Power Reduction (MPR) for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156046 | 2320 | 1 | Additions to test case 6.3.2E – Minimum Output Power for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156047 | 2332 | 1 | Additions to test case 6.3.5E.3 – Aggregate power control tolerance for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156048 | 2340 | 1 | Additions to test case 6.6.1E – Occupied bandwidth for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156049 | 2341 | 1 | Additions to test case 6.6.2.1E – Spectrum Emission Mask for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156057 | 2193 | 1 | New CA 2UL Tx test case 6.5.1A.3 Frequency Error for CA (intra-band non-contiguous DL CA and UL CA) | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156058 | 2291 | 1 | New TC of A-MPR for 2UL inter-band CA (6.2.4A.2) | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156059 | 2293 | 1 | New TC of Additional spurious emissions for intra-band non-contiguous CA 6.6.3.3A.3 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156062 | 2166 | 1 | Addition of Minimum test time for LTE Type C performance in Annex G | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156063 | 2297 | 1 | Addition of new TC 7.4_H Maximum input level for SCE-L1 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156064 | 2298 | 1 | Addition of new TC 7.4A.1_H Maximum input level for SCE-L1 for CA (intra-band contiguous DL CA and UL CA) | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156065 | 2299 | 1 | Addition of new TC 7.4A.2_H Maximum input level for SCE-L1for CA (intra-band contiguous DL CA without UL CA) | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156066 | 2300 | 1 | Addition of new TC 7.4A.5_H Maximum input level for SCE-L1 for CA (3DL CA without UL CA) | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156089 | 2185 | 1 | LTE Type B performance requirements - Introduction of new test cases 8.2.1.2.5 and 8.2.1.2.6 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156090 | 2186 | 1 | LTE Type B performance requirements - Introduction of new test cases 8.2.2.2.6 and 8.2.2.2.7 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156091 | 2187 | 1 | LTE Type B performance requirements - Introduction of new test cases 9.3.8.1.1 and 9.3.8.1.2 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156092 | 2188 | 1 | LTE Type B performance requirements - Adding six NAICS test cases into Annex F and G | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156095 | 2155 | 1 | Update Minimum requirements for test case 6.2.3A.1 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156096 | 2227 | 1 | Updates of test configuration for OBW for intra-band contiguous UL CA | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156097 | 2268 | 1 | CA RF: Minor corrections to section 6.6.3.3A | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156098 | 2292 | 1 | Revise Configured UE transmitted Output Power for 2UL intra-band contiguous CA test case 6.2.5A.1 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156100 | 2167 | 1 | Deleting redundant and outdated Demod scenario in Minimum test time | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156101 | 2258 | 1 | Clarification of DL configuration for wideband | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156102 | 2260 | 1 | Correction to Enhanced Performance Type A performance test cases | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156103 | 2277 | 1 | Correction and addition of minimum test time for several tests | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156104 | 2284 | 1 | Correction to test cases of the section of 9.5.1 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156110 | 2190 | 1 | Introduction of new test case 8.9.2.1.1 MTC | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156113 | 2301 | 1 | Addition of new TC 8.2.1.4.1_H FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 2x2 for SCE-L1 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156114 | 2302 | 1 | Addition of new TC 8.2.2.4.1_H TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 2x2 for SCE-L1 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156115 | 2303 | 1 | Addition of new TC 8.3.1.1.1_H FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 without a simultaneous transmission for SCE-L1 for eDL-MIMO | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156116 | 2304 | 1 | Addition of new TC 8.3.2.1.2_H TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 without a simultaneous transmission for eDL-MIMO for SCE-L1 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156117 | 2305 | 1 | Addition of new TC 8.7.1.1_H FDD sustained data rate performance for SCE-L1 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156118 | 2306 | 1 | Addition of new TC 8.7.1.1_H.2 FDD Sustained data rate performance for CA for SCE-L1 (2DL CA) | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156119 | 2307 | 1 | Addition of new TC 8.7.1.1_H.3 FDD Sustained data rate performance for CA for SCE-L1 (3DL CA) | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156120 | 2308 | 1 | Addition of new TC 8.7.2.1_H.1 TDD sustained data rate performance for SCE-L1 (Single Carrier) | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156121 | 2309 | 1 | Addition of new TC 8.7.2.1_H.2 TDD sustained data rate performance for CA for SCE-L1 (2DL CA) | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156122 | 2310 | 1 | Addition of new TC 8.7.2.1_H.3 TDD Sustained data rate performance for CA for SCE-L1 (3DL CA) | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156124 | 2312 | 1 | Addition of new TC 8.7.5.1_H.2 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (3DL CA) | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156125 | 2313 | 1 | Addition of new TC 8.7.5.2_H.1 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (2DL CA) | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156126 | 2314 | 1 | Addition of new TC 8.7.5.2_H.2 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (3DL CA) | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156130 | 2192 | 2 | New CA 2UL Tx test case 6.3.5A.3.3 of Aggregate Power Control Tolerance for CA (intra-band non-contiguous DL CA and UL CA) | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156131 | 2290 | 2 | New TC of Aggregate Power Control Tolerance for 2UL inter-band CA(6.3.5A.3.2) | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156133 | 2294 | 1 | Correction to the applicable test release of 1.4MHz Bandwidth MBMS test | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156134 | 2164 | 1 | Update to in-band Emissions Limits for less than 1GHz Carrier Frequencies | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156183 | 2311 | 1 | Addition of new TC 8.7.5.1_H.1 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (2DL CA) | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156071 | 2153 | 1 | New TC: Maximum Power Reduction (MPR) for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM | 12.8.0 | 13.0.0 |
| 2015-12 | RAN#70 | R5-156073 | 2194 | 1 | New TC: 6.2.3_3 MPR for UL 64QAM | 12.8.0 | 13.0.0 |
| 2015-12 | RAN#70 | R5-156074 | 2195 | 1 | New TC: 6.2.3_4 MPR for Multi-Cluster PUSCH with UL 64QAM | 12.8.0 | 13.0.0 |
| 2015-12 | RAN#70 | R5-156075 | 2196 | 1 | New TC: 6.2.4_2 AMPR for UL 64QAM | 12.8.0 | 13.0.0 |
| 2015-12 | RAN#70 | R5-156076 | 2197 | 1 | New TC: 6.5.2.1_1 EVM for UL 64QAM | 12.8.0 | 13.0.0 |
| 2015-12 | RAN#70 | R5-156077 | 2198 | 1 | New TC: 6.5.2A.1.1_1 EVM for CA with UL 64QAM | 12.8.0 | 13.0.0 |
| 2015-12 | RAN#70 | R5-156078 | 2199 | 1 | New TC: 6.6.2.2_1 Additional Spectrum Emission Mask for UL 64QAM | 12.8.0 | 13.0.0 |
| 2015-12 | RAN#70 | R5-156079 | 2200 | 1 | New TC: 6.6.2.3_3 ACLR for UL 64QAM | 12.8.0 | 13.0.0 |
| 2015-12 | RAN#70 | R5-156080 | 2201 | 1 | New TC: 6.6.2.3_4 ACLR for Multi-Cluster PUSCH with UL 64QAM | 12.8.0 | 13.0.0 |
| 2015-12 | RAN#70 | R5-156081 | 2202 | 1 | New TC: 6.6.3.3_1 Additional spurious emissions for UL 64QAM | 12.8.0 | 13.0.0 |
| 2015-12 | RAN#70 | R5-156082 | 2203 | 1 | Addition of RMCs for UL 64QAM | 12.8.0 | 13.0.0 |
| 2015-12 | RAN#70 | R5-156083 | 2204 | 1 | Introduction of uncertainties and TTs in Annex F for UL 64QAM | 12.8.0 | 13.0.0 |
| 2015-12 | RAN#70 | R5-156084 | 2285 | 1 | New TC: A-MPR for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM | 12.8.0 | 13.0.0 |
| 2015-12 | RAN#70 | R5-156085 | 2286 | 1 | New TC: A-SEM for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM | 12.8.0 | 13.0.0 |
| 2015-12 | RAN#70 | R5-156086 | 2287 | 1 | New TC: Additional spurious emissions for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM | 12.8.0 | 13.0.0 |
| 2015-12 | RAN#70 | R5-156188 | 2154 | 2 | New TC: Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM | 12.8.0 | 13.0.0 |
| 2016-01 | RAN#70 | R5-156099 | 2324 | 1 | introduction of R5-156099 "Introduction of new A-MPR test with PUSCH frequency hopping" with the test case renumbered to 6.2.4_3 after working group agreement to avoid duplication | 13.0.0 | 13.0.1 |
| 2016-03 | RAN#71 | R5-160025 | 2347 | - | Update to NAICS test cases to release NAICS configuration | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160026 | 2348 | - | Test Tolerances for FDD PDSCH Transmit Diversity 2x2 with TM2 Interference Model– Enhanced Performance Requirement Type B (8.2.1.2.5) | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160027 | 2349 | - | Test Tolerances for FDD PDSCH Transmit Diversity 2x2 with TM9 Interference Model– Enhanced Performance Requirement Type B (8.2.1.2.6) | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160028 | 2350 | - | Test Tolerances for TDD PDSCH Transmit Diversity 2x2 with TM2 Interference Model– Enhanced Performance Requirement Type B (8.2.2.2.6) | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160029 | 2351 | - | Test Tolerances for TDD PDSCH Transmit Diversity 2x2 with TM9 Interference Model– Enhanced Performance Requirement Type B (8.2.2.2.7) | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160030 | 2352 | - | Test Tolerances FDD and TDD CQI reporting under fading conditions (PUCCH) TM4 - Enhanced Receiver Type B (9.3.8.1.1 and 9.3.8.1.2) | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160067 | 2362 | - | Correction to UE output power of Tx test cases 6.5.2.3 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160068 | 2363 | - | Correction to some incorrect sentences of Tx test case 6.5.2A.3.1 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160106 | 2368 | - | Update of Test Tolerance for eDL MIMO enhancement test cases | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160124 | 2371 | - | Correction to test case 8.7.1.1_A.1 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160125 | 2372 | - | Correction to test case 9.3.2.1.1 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160139 | 2375 | - | Test Tolerances for 9.6.1.3.1+9.6.1.3.2+9.6.1.4.1+9.6.1.4.2 2DL-3DL CA | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160141 | 2376 | - | TTs to Annex F for 2DL and 3DL CA | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160155 | 2382 | - | Test Tolerances for 9.3.7.1+9.3.7.2 eDL MIMO enhancement | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160159 | 2383 | - | Test Tolerances for 9.4.1.4.1+9.4.1.4.2+9.4.2.3.3+9.4.2.3.4 eDL MIMO enhancement | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160162 | 2385 | - | Corrections to eDL MIMO enhancements Annex B to include High XP 4x2 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160164 | 2386 | - | Corrections to EVM test case 6.5.2A.1.1 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160165 | 2387 | - | Update of uncertainty and TT references for TC 6.2.4E and 6.2.5E | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160169 | 2390 | - | Addition of 4 Rx part to Diversity characteristics | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160203 | 2391 | - | Revise Power Control Relative power tolerance for CA (intra-band contiguous DL CA and UL CA) test case 6.3.5A.2.1 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160281 | 2419 | - | Addition of delta TIB for the new CA configuration CA_3A-41A in sub-clause 6.2.5 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160334 | 2423 | - | Updates of Spurious emission band UE co-existence for CA (inter-band DL CA and UL CA) | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160337 | 2425 | - | Annex F update for frequency error for inter-band 2UL CA | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160338 | 2426 | - | Correction and updates to test requirements of Reference sensitivity for inter-band 2DL CA | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160344 | 2428 | - | Updates of some Rel-13 defined CA combinations to minimum requirements of Reference sensitivity for CA | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160345 | 2429 | - | Updates of delta TIB,c values and table references for delta TIB,c | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160377 | 2437 | - | R10 Remove editor's note for CA_39C spurious emission test | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160441 | 2439 | - | Update to MTC test case 8.9.2.1.1 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160442 | 2440 | - | Update to MTC RF test applicability for Editor’s Note | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160459 | 2443 | - | Rel-13 Test point reduction for UL 64QAM multi-cluster MPR and ACLR tests | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160461 | 2445 | - | Rel-13 Test point reduction for UL 64QAM AMPR and ASEM tests | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160462 | 2446 | - | Rel-13 Test point reduction for UL 64QAM CA MPR and ACLR tests | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160464 | 2448 | - | Rel-13 Test point reduction for UL 64QAM CA AMPR, ASEM and ASE tests | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160482 | 2451 | - | Correction to the minimum test time reference in TDD/FDD CA performance | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160483 | 2452 | - | Correction to UL CA Tx tests | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160484 | 2453 | - | Clean up on performance test cases | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160485 | 2454 | - | Correction to 9.5.5.1_F.1 and 9.5.5.1_F.2 for RI Reporting test with CoMP | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160486 | 2455 | - | Correction to 8.2.2.2.4 and 8.2.2.4.3 for Enhanced Performance Type A | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160487 | 2456 | - | Correction to CQI reporting tests for Enhanced Performance Type A | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160488 | 2457 | - | Correction to CQI and RI reporting tests for feICIC | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160490 | 2459 | - | Correction to PCFICH/PDCCH and PHICH Tx Diversity for feICIC | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160491 | 2460 | - | Correction to G.5.4 for minimum test time | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160504 | 2462 | - | New CA band combination CA_41A_42C - Updates of 6.2.5.3 Delta TIB | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160506 | 2463 | - | New CA band combination CA_41A_42C - Updates of 7.3.3 Delta RIB | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160527 | 2468 | - | Addition of operating band and channel assignments for the new Intel assigned CA configuration | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160569 | 2477 | - | Correction to PMI Reporting for eDL-MIMO | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160578 | 2480 | - | Minor corrections to applicabilities of TCs 8.2.2.2.3_E.1 & 8.2.2.3.3_E.1 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160580 | 2482 | - | Correction to PDCP SDU size in TC 8.7.2.1 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160582 | 2484 | - | CA RF: Corrections to Notes in TC 7.3A.3 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160583 | 2485 | - | CA RF: Corrections to Notes in TC 7.3A.5 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160584 | 2486 | - | CA RF: Minor correction to Note in TC 7.3A.6 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160586 | 2488 | - | Corrections to DL 256QAM SDR Test Cases | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160587 | 2489 | - | Corrections to TDD FDD CA SDR Test Cases | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160588 | 2490 | - | Addition of Maximum Wgap definition | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160590 | 2492 | - | Corrections to TDD FDD CA Closed Loop Spatial Multiplexing TCs | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160592 | 2494 | - | Corrections to TDD FDD CA Test Applicability tables in section 8.1.2.3B | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160622 | 2497 | - | Addition of delta RIB for CA_3A-41A in TC 7.3.3 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160681 | 2516 | - | Introduction of test case 6.6.2.3A.3 - Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous DL CA and UL CA) | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160686 | 2519 | - | Add reference to TR 36.905 in 36.521-1 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160801 | 2479 | 1 | Annex F: Corrections for intra-band contiguous UL CA Test Cases | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160802 | 2483 | 1 | Minor correction to TC 6.6.3.2 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160803 | 2487 | 1 | Removal of editor’s notes in TC 6.5.2A.2.1 & 6.5.2A.3.1 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160804 | 2466 | 1 | Correction to UE category information in test case 8.2.1.1.1 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160805 | 2467 | 1 | Correction to test case applicability for test case 9.2.1.3_C.1 and 9.2.1.4_C.1 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160811 | 2430 | 1 | Updates of delta RIB,c values and table references for delta RIB,c | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160812 | 2431 | 1 | Addition of CA_19A-28A and CA_21A-42A to refsens for inter-band 2DL CA | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160813 | 2432 | 1 | Updates of test points and test requirements of some Rel-13 defined CA combinations for Reference sensitivity for 3DL CA | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160814 | 2469 | 1 | Addition of CA configurations and bandwidth combination sets for the new CA configuration | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160815 | 2496 | 1 | Add delta TIB for the new CA_20A-67A configuration in sub-clause 6.2.5. | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160819 | 2400 | 1 | Addition of new TC 7.4A.3_H Maximum input level for CA (inter-band DL CA without UL CA) for 256QAM in DL | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160820 | 2401 | 1 | Addition of new TC 7.4A.4_H Maximum input level for CA (intra band non-contiguous DL CA without UL CA) for 256QAM in DL | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160825 | 2436 | 1 | Rel-10 Test point reduction for UL 64QAM multi-cluster MPR and ACLR tests | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160828 | 2422 | 1 | Updates to TM3 TDD-FDD Soft buffer 2DLCA test cases | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160829 | 2493 | 1 | Corrections to TDD FDD CA TCs 9.6.1.3.1 and 9.6.1.4.1 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160831 | 2520 | 1 | Test points for CA_4A-4A-13A in Reference sensitivity test case | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160835 | 2379 | 1 | Addition of UL CA capability for CA_4A-4A in Table 5.4.2A.1-3 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160836 | 2380 | 1 | Correction for table references in TC 6.6.3.2A.1 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160837 | 2381 | 1 | Renaming of Table 6.6.3.2A-3a | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160838 | 2409 | 1 | Update to Annex F for TC 6.3.5A.1.2, 6.3.5A.1.3 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160839 | 2411 | 1 | Update to TC 6.3.5A.1.2 Power Control Absolute Power Tolerance for CA (inter-band DL CA and UL CA). | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160840 | 2412 | 1 | Update to TC 6.3.5A.1.3 Power Control Absolute power tolerance for CA (intra-band non-contiguous DL CA and UL CA). | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160841 | 2416 | 1 | Correction to test case 6.2.5A.3 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160845 | 2424 | 1 | Addition of new test case for additional spurious for inter-band 2UL CA and updates for A-MPR for inter-band 2UL CA | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160846 | 2518 | 1 | Introduction of test case 7.3A.7 - Reference sensitivity level for CA (intra-band non-contiguous DL CA and UL CA) | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160849 | 2470 | 1 | New TC: FDD CQI Reporting under fading conditions - PUCCH 1-1 (CSI Reference Symbol) TM9 - Enhanced Receiver Type B | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160850 | 2471 | 1 | New TC: TDD CQI Reporting under fading conditions - PUCCH 1-1 (CSI Reference Symbol) TM9 - Enhanced Receiver Type B | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160852 | 2393 | 1 | Update to TC 8.9.1.1.3 FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category 0 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160853 | 2394 | 1 | Update to TC 8.9.1.2.1 TDD PDSCH Transmit Diversity for UE category 0 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160854 | 2395 | 1 | Update to TC 8.9.1.2.3 TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category 0 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160855 | 2408 | 1 | Update to TC 9.7.2.2 TDD CQI reporting definition under fading conditions for UE category 0 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160856 | 2420 | 1 | Update to TC 9.7.2.1 FDD and Half duplex FDD CQI reporting definition under fading conditions for UE category 0 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160858 | 2498 | 1 | Additions to test case 6.6.2.2E – Additional Spectrum Emission Mask for UE category 0 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160859 | 2500 | 1 | Additions to test case 6.6.3E.3 – Additional spurious emissions for UE category 0 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160860 | 2501 | 1 | Additions to test case 6.6.2.3E – Adjacent Channel Leakage power Ratio for UE category 0 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160861 | 2504 | 1 | Additions to test case 6.3.4E.2 – PRACH and SRS ON/OFF time mask for UE category 0 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160862 | 2506 | 1 | Additions to test case 6.7E – Transmit intermodulation for UE category 0 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160863 | 2508 | 1 | Additions to test case 6.5.2.1E.2 – PUSCH-EVM with exclusion period for UE category 0 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160864 | 2510 | 1 | Additions of overlying section 6.6.3E-Spurious emission for UE category 0 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160865 | 2511 | 1 | Additions of overlying sections 6.3.5C, 6.3.5D and 6.3.5E Power Control for UE category 0 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160866 | 2513 | 1 | Additions to test case 6.2.3E - Maximum Power Reduction (MPR) for UE category 0 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160867 | 2515 | 1 | Additions of overlying section 6.3.4E-ON/OFF time mask for UE category 0 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160868 | 2507 | 1 | Additions to test case 6.6.3E.2 –Spurious emission band UE co-existence for UE category 0 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160869 | 2505 | 1 | Additions to test case 6.6.3E.1 – Transmitter Spurious emissions for UE category 0 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160870 | 2370 | 1 | Updates and test tolerance to single antenna FDD-TDD CA test cases | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160877 | 2384 | 1 | TTs to Annex F for eDL MIMO enhancement | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-160878 | 2392 | 1 | Update to Annex F for TC 8.9.1.1.3, 8.9.1.2.1, 8.9.1.2.3, 9.7.2.1, 9.7.2.2 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161001 | 2495 | 1 | Addition of B66 in Frequency bands and channel arrangement. | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161004 | 2427 | 1 | Updates of some Rel-13 defined CA combinations to chapter 5 of TS 36.521-1 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161005 | 2449 | 1 | New ProSe test case: Demodulation of PSDCH for ProSe direct discovery | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161006 | 2450 | 1 | New ProSe test case: Maximum input level test | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161008 | 2398 | 1 | Updates to TC 8.7.1.1_A.2 FDD Sustained data rate performance for CA (3DL CA) | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161009 | 2421 | 1 | Updates to TM3 TDD-FDD with TDD Pcell 2DLCA and 3DLCA test cases | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161010 | 2491 | 1 | Corrections to TDD FDD CA Open Loop Spatial Multiplexing TCs | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161012 | 2502 | 1 | 36.521-1 Sensitivity test points revision for 3DL 1A-3A-7A | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161014 | 2503 | 1 | 36.521-1 Sensitivity test points revision for 3DL 1A-3A-20A | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161017 | 2417 | 1 | Update to TC 6.2.4A.2 A-MPR for 2UL inter-band CA | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161019 | 2356 | 1 | New CA 2UL Tx test case 6.5.2A.1.3 Error Vector Magnitude (EVM) for CA (intra-band non-contiguous DL CA and UL CA) | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161022 | 2359 | 1 | New CA 2UL Tx test case 6.5.2A.3.2 In-band emissions for non allocated RB for CA (inter-band DL CA and UL CA) | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161023 | 2360 | 1 | New CA 2UL Tx test case 6.5.2A.3.3 In-band emissions for non allocated RB for CA (intra-band non-contiguous DL CA and UL CA) | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161024 | 2361 | 1 | New CA 2UL Tx test case 6.6.3.1A.3 Transmitter Spurious emissions for CA (intra-band non-contiguous DL CA and UL CA) | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161025 | 2377 | 1 | Addition of new LTE_CA_Rel12_2UL test case 6.6.3.2A.3 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161026 | 2378 | 1 | Completion of new LTE_CA_Rel12_2UL Test Case 6.7A.2 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161028 | 2517 | 1 | Introduction of test case 6.6.2.2A.2 - Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA) | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161029 | 2407 | 1 | Updates to Annex A for 256QAM test | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161030 | 2403 | 1 | Addition of new TC 9.2.1.7 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 for 256QAM in DL | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161031 | 2404 | 1 | Addition of new TC 9.2.1.8 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 for 256QAM in DL | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161032 | 2405 | 1 | Addition of new TC 9.3.1.2.3 FDD CQI Reporting under fading conditions – PUSCH 3-1 for 256QAM in DL | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161033 | 2406 | 1 | Addition of new TC 9.3.1.2.4 TDD CQI Reporting under fading conditions – PUSCH 3-1 for 256QAM in DL | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161034 | 2399 | 1 | Updates to 7.4A.1_H and 7.4A.2_H Maximum input level for intra-band contiguous CA for 256QAM in DL | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161035 | 2402 | 1 | Updates to TC 7.4A.5_H Maximum input level for CA (3DL CA without UL CA) for 256QAM in DL | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161039 | 2353 | 1 | LTE Type B performance requirements - Introduction of new test cases 8.2.1.4.4 and 8.2.2.4.5 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161040 | 2354 | 1 | LTE Type B performance requirements - Adding test cases 8.2.1.4.4 and 8.2.2.4.5 into Annex F and G | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161041 | 2433 | 1 | Test configuration correction for AMPR tests | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161042 | 2434 | 1 | Test configuration correction for ASE tests | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161043 | 2435 | 1 | Test configuration correction for ASEM tests | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161044 | 2438 | 1 | Test configuration correction for UL 64QAM AMPR tests | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161045 | 2441 | 1 | Test configuration correction for UL 64QAM ASE tests | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161046 | 2442 | 1 | Test configuration correction for UL 64QAM ASEM tests | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161047 | 2444 | 1 | Rel-8 Test point reduction for UL 64QAM AMPR, ASEM and ASE tests | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161056 | 2410 | 1 | Adding new clause 6.3.5A.1.0 and 6.3.5A.2.0 to define the minimum conformance requirements. | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161059 | 2465 | 1 | New CA band combination CA_41A-42C - Updates of 7.3A Refsens | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161061 | 2447 | 1 | Rel-10 Test point reduction for UL 64QAM CA AMPR, ASEM and ASE tests | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161062 | 2472 | 1 | Correction to the UL CA Frequency Error Testcase | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161064 | 2522 | 1 | Correction to A-MPR test with frequency hopping | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161065 | 2415 | 1 | Correction of Table number of DeltaRib reference in CA receiver RF tests | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161073 | 2367 | 1 | Update of test applicability for eDL MIMO enhancement TCs | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161085 | 2514 | 1 | 36.521-1 Editorial corrections | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161086 | 2524 | 1 | Addition frequency bands and channel arrangement for B67 and CA_20-67 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161103 | 2355 | 2 | New CA 2UL Tx test case 6.5.2A.1.2 Error Vector Magnitude (EVM) for CA (inter-band DL CA and UL CA) | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161104 | 2357 | 2 | New CA 2UL Tx test case 6.5.2A.2.2 Carrier leakage for CA (inter-band DL CA and UL CA) | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161105 | 2358 | 2 | New CA 2UL Tx test case 6.5.2A.2.3 Carrier leakage for CA (intra-band non-contiguous DL CA and UL CA) | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161107 | 2389 | 2 | Reference sensitivity level with 4 Rx antenna ports | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161109 | 2509 | 2 | 36.521-1 Transmitter tests for Band 66 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161110 | 2512 | 1 | 36.521-1 Receiver tests for Band 66 | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161111 | 2373 | 1 | Correction to power level for dual layer TM9 demodulation tests | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161112 | 2458 | 2 | Correction to TDD CQI Reporting for feICIC | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161113 | 2461 | 2 | Correction to UL 64 QAM RMC | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161114 | 2475 | 2 | Correction to CQI reports in CoMP fading test | 13.0.1 | 13.1.0 |
| 2016-03 | RAN#71 | R5-161115 | 2476 | 2 | Correction to test system uncertainties for Rx 7.6.3 | 13.0.1 | 13.1.0 |
| 2016-06 | RAN#72 | R5-162088 | 2527 | - | Test Tolerances for FDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x2 with TM4 Interference Model– Enhanced Performance | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162090 | 2528 | - | Test Tolerances for TDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x2 with TM4 Interference Model– Enhanced Performance | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162169 | 2533 | - | Corrections to Annex F.1.2 and F.3.2 for Pcmax test case | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162189 | 2536 | - | Additions to test case 6.3.5E.3 – Aggregate power control tolerance for UE category 0 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162191 | 2537 | - | Additions to test case 6.3.5E.2 – Power Control Relative power tolerance for UE category 0 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162192 | 2538 | - | Editorial correction in Reference sensitivity QPSK PREFSENS four antenna ports Table number | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162195 | 2539 | - | Addition In-band blocking with 4 Rx antenna ports test case | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162210 | 2542 | - | Addition of UL CA capability for CA_4A-5A in Table 5.4.2A.1-2 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162211 | 2543 | - | New test points for CA_4A-5A (2DL-2UL) in sec. 6.2 and sec. 6.3 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162212 | 2544 | - | New test points for CA_4A-5A (2DL-2UL) in sec. 6.5 and sec. 6.6 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162215 | 2545 | - | Update of requirements in TC 6.2.2A.2 for CA_4A-5A (2DL-2UL) | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162235 | 2557 | - | Introduction of ACLR for intra-band non-contiguous UL CA for UL 64QAM | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162241 | 2563 | - | New test case: Maximum sidelink processes for ProSe Direct discovery | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162247 | 2569 | - | Update of TC 7.3A.6 for CA_4A-5A (2DL-2UL) | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162248 | 2570 | - | New D2D Tx test case 6.5.1D.2 Frequency error for ProSe Direct Communication | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162249 | 2571 | - | Update of Inter-band E-UTRA ProSe / E-UTRA operating bands to chapter 5 of TS 36.521-1 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162251 | 2572 | - | Update to CA 2UL Tx test case 6.5.1A.3 Frequency Error for CA (intra-band non-contiguous DL CA and UL CA) | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162296 | 2590 | - | New TC: FDD PDSCH Closed Loop Single-layer Spatial Multiplexing on antenna ports 7 or 8 with TM9 Interference Model - Enhanced Performance Requirement Type B | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162303 | 2593 | - | RF: Wrong Number of PDCCH symbols in TC 8.2.1.4.1_H | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162304 | 2594 | - | RF: Corrections to RMCs in Annex A | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162306 | 2595 | - | RF: Correction to RMC for UE Category 1 in CSI tests | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162308 | 2597 | - | RF: Pb setting in power imbalance TCs | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162309 | 2598 | - | RF: UE Categories in FDD-TDD 2DL joint operation CSI test cases | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162324 | 2605 | - | Introduction of TDD PCFICH/PDCCH Single-antenna Port Performance 1x4 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162340 | 2608 | - | Added Channel BW clause for Dual Connectivity | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162343 | 2609 | - | Introduction of Spurious Response for ProSe | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162344 | 2610 | - | Introduction of In-band blocking for ProSe | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162345 | 2611 | - | Introduction of Out-of-band blocking for ProSe | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162346 | 2612 | - | Annex F: Addition of chapter 7 ProSe TCs | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162350 | 2615 | - | UL CA: Corrections to 6.5.2A.x inter-band and non-contiguous TCs | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162354 | 2618 | - | Correction to UL64QAM A-MPR and ASE tests | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162355 | 2619 | - | Correction to TC 8.2.1.3.1A_A.1 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162358 | 2622 | - | Annex G: Corrections to test times for joint operation TCs | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162393 | 2625 | - | Addition CA_66A-66A in test case 7.4A.4_H | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162420 | 2628 | - | Addition of CA_1A-3A-7A in 36.521-1 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162421 | 2629 | - | Addition of CA_3A-7A-8A in 36.521-1 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162463 | 2633 | - | Addition of Band 45 to Section 5 of 36.521-1 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162464 | 2634 | - | Addition of Band 45 in maximum output power test cases | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162465 | 2635 | - | Addition of Band 45 in MPR test cases | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162467 | 2637 | - | Addition of Band 45 in Refsens test cases | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162468 | 2638 | - | Addition of Band 45 in in-band blocking test cases | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162469 | 2639 | - | Addition of Band 45 in out-of-band blocking test cases | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162475 | 2640 | - | Addition of output power configuration for CA_3C-5A to Section 6.2.5 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162486 | 2644 | - | Updates of test configuration for Additional spurious emissions for 2UL inter-band CA | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162490 | 2645 | - | New CA band combinations CA_28A-41A and CA_28A-41C - Updates of 5.4.2A Channel bandwidth | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162491 | 2646 | - | New CA band combinations CA_28A-41A and CA_28A-41C - Updates of 6.2.5.3 Delta TIB | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162492 | 2647 | - | New CA band combinations CA_28A-41A and CA_28A-41C - Updates of 7.3.3 Delta RIB | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162495 | 2649 | - | New CA band combinations CA_28A-42A and CA_28A-42C - Updates of 5.4.2A Channel bandwidth | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162496 | 2650 | - | New CA band combinations CA_28A-42A and CA_28A-42C - Updates of 6.2.5.3 Delta TIB | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162497 | 2651 | - | New CA band combinations CA_28A-42A and CA_28A-42C - Updates of 7.3.3 Delta RIB | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162499 | 2653 | - | New CA band combinations CA_41C-42A and CA_41C-42C - Updates of 5.4.2A Channel bandwidth | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162500 | 2654 | - | New CA band combinations CA_41C-42A and CA_41C-42C - Updates of 6.2.5.3 Delta TIB | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162501 | 2655 | - | New CA band combinations CA_41C-42A and CA_41C-42C - Updates of 7.3.3 Delta RIB | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162502 | 2656 | - | New CA band combinations CA_41C-42A - Updates of 7.3A Refsens | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162527 | 2661 | - | Correction of editor's note for CA_41A-42C | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162528 | 2662 | - | Minor correction to TC 8.2.1.4.2_A.1 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162542 | 2666 | - | Updates of delta TIB,c values for some CA configurations | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162549 | 2668 | - | Correction to test case 9.3.2.2.2_D | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162550 | 2669 | - | Correction to test applicability in test case 8.7.5.1.1 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162553 | 2671 | - | Correction to test case 9.3.3.1.1. | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162563 | 2676 | - | Introduction of new Rx test case Maximum input level with 4 Rx antenna ports | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162567 | 2678 | - | Addition of 2x1 antenna correlation matrices for UE category 0 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162568 | 2679 | - | Updates to test case 8.9.1.1.1 - FDD PDSCH Transmit Diversity 2x1 for UE category 0 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162569 | 2680 | - | Updates to test case 9.7.1.1 - FDD and Half duplex FDD CQI reporting definition under AWGN conditions for UE category 0 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162593 | 2688 | - | Correction to CQI tests for Enhanced Performance Type A | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162594 | 2689 | - | Correction to TDD FDD CA CQI Reporting tests | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162596 | 2691 | - | Correction to demodulation test cases for CoMP in 8.3.1.3.2_F and 8.3.2.4.2_F | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162597 | 2692 | - | Correction to PDCCH Aggregation Level for SDR test | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162605 | 2695 | - | Update of refsens test case 7.3A.3 for CA_20A-67A | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162647 | 2700 | - | Re-Addition of new LTE_CA_Rel12_2UL test case 6.6.3.2A.3 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162661 | 2704 | - | Band 65 introduction to Chapter 7 in 36.521-1 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162802 | 2546 | 1 | Introduction of MPR for inter-band DL CA and UL CA for UL 64QAM | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162803 | 2547 | 1 | Introduction of MPR for intra-band non-contiguous CA and UL CA for UL 64QAM | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162804 | 2553 | 1 | Rel-11 Test point reduction for intra-band non-contiguous UL CA MPR and ACLR tests | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162805 | 2554 | 1 | Rel-11 Test point reduction for inter-band UL CA AMPR, ASEM and ASE tests | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162806 | 2555 | 1 | R8 - Modification of test bandwidths in AMPR, ASEM and ASE test | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162807 | 2556 | 1 | R13 - Modification of test bandwidths in AMPR and ASE for UL 64QAM test | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162808 | 2616 | 1 | UL CA: Corrections to TC 6.5.2A.1.1 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162810 | 2685 | 1 | Correction to 6.2.4 A-MPR test for Band 12 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162811 | 2535 | 1 | Corrections to CA_41A-41C and CA_41C-41A tables in clauses 7.3A.5, 7.4A.5, 7.5A.5, 7.6.1A.5, 7.6.2A.5, 7.6.3A.5 and 7.8.1A.5 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162812 | 2665 | 1 | Correction of refsens test case 7.3A.3 for CA_20A-32A | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162813 | 2674 | 1 | New CA band combination CA_8A-40A – Updates of 7.3.3 Delta RIB | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162814 | 2684 | 1 | New CA band combination CA_8A-40A – Updates of 7.3A Refsens | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162815 | 2686 | 1 | Correction to reference sensitivity test cases | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162816 | 2687 | 1 | Clarification of interferer frequencies in ACS | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162817 | 2621 | 1 | Corrections to chapter 8 joint operation TCs | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162818 | 2670 | 1 | Correction to test case 8.2.3.2.2A | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162819 | 2690 | 1 | Additional test point for 3DL CA Performance and CQI test cases | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162820 | 2693 | 1 | Correction to TDD FDD CA performance test cases for FDD PCell(2DL and 3DL) | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162821 | 2694 | 1 | Correction to TDD FDD CA performance test cases for TDD PCell TCs(2DL) | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162832 | 2623 | 1 | Corrections to 256QAM SDR joint operation TCs | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162837 | 2559 | 1 | New test case: 2x4 PDSCH TM9 single layer TypeA | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162838 | 2530 | 1 | Addition of CA_4A-12A Bandwidth combination set5 information | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162839 | 2567 | 1 | Update of TC 6.6.3.1A.2 for CA_4A-5A (2DL-2UL) | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162840 | 2584 | 1 | Updates TC7.3 for new Rel-13 defined CA combinations to minimum requirements of Reference sensitivity | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162841 | 2585 | 1 | Updates TC7.3A for new Rel-13 defined CA combinations to minimum requirements of Reference sensitivity for CA | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162842 | 2586 | 1 | Updates of some Rel-13 defined CA combinations to operating bands of TS 36.521-1 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162843 | 2587 | 1 | Updates TC6.2.5 for delta TIB,c values with new Rel-13 CA band combination | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162844 | 2627 | 1 | Addition of CA_4A-4A BW set 1 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162846 | 2531 | 1 | Addition of Rel-13 CA combination CA_3A-41A to refsens for inter-band 2DL CA | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162847 | 2710 | 1 | Introduction of new TC8.2.1.4.2_A.3 4DL CA | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162848 | 2719 | 1 | Addition of definition of CA capability with 4DL & 5DL CCs | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162849 | 2720 | 1 | Addition of Operating bands for CA in clause 5.2A | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162851 | 2560 | 1 | New test case: 4layer PDSCH CLSM | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162852 | 2561 | 1 | New test case: 4layer PDSCH TM9 SM | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162853 | 2562 | 1 | New test case: 3layer PDSCH OLSM | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162854 | 2573 | 1 | New Performance test case 8.10.4.1.1 FDD distributed EPDCCH performance 2x4 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162855 | 2574 | 1 | New Performance test case 8.10.4.1.2 TDD distributed EPDCCH performance 2x4 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162856 | 2575 | 1 | New Performance test case 8.10.4.2.1 FDD localized EPDCCH performance with TM9 2x4 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162857 | 2576 | 1 | New Performance test case 8.10.4.2.2 TDD localized EPDCCH performance with TM9 2x4 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162858 | 2602 | 1 | Introduction of FDD PCFICH/PDCCH Single-antenna Port Performance 1x4 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162859 | 2603 | 1 | Introduction of FDD PCFICH/PDCCH Transmit Diversity 2x4 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162860 | 2606 | 1 | Introduction of TDD PCFICH/PDCCH Transmit Diversity 2x4 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162861 | 2607 | 1 | Introduction of TDD PCFICH/PDCCH Transmit Diversity 4x4 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162862 | 2675 | 1 | Addition of 4Rx antenna port feature into Annex I | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162863 | 2677 | 1 | Introduction of new Rx test case Adjacent Channel Selectivity (ACS) with 4 Rx antenna ports | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162865 | 2577 | 1 | Rearrangement of TC 6.6.2.3A.1_1 and TC 6.6.2.3A.2 and corrections to TC 6.6.2.3A.2 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162866 | 2614 | 1 | UL CA: Corrections to intra-band non-contiguous test configuration tables | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162867 | 2725 | 1 | Updates of test configuration for EVM and Carrier leakage test cases of 2UL CA | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162868 | 2643 | 1 | Addition of minimum requirement tables to clause 6.3.5A.1.0 and 6.3.5A.2.0 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162869 | 2681 | 1 | Updates to test case 6.6.2.2A.2 - Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA) | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162870 | 2699 | 1 | Re-submission of New CA 2UL Tx test case 6.5.2A.1.3 Error Vector Magnitude (EVM) for CA (intra-band non-contiguous DL CA and UL CA) | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162872 | 2724 | 1 | Additions to test case 6.6.3.1A.2 – Transmitter Spurious emissions for CA (inter-band DL CA and UL CA) | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162875 | 2696 | 1 | Adding operating band definitions for MTC and eMTC | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162877 | 2564 | 1 | New test case: PSSCH performance for ProSe Direct communication | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162878 | 2565 | 1 | New test case: Maximum sidelink processes for ProSe Direct communication | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162879 | 2722 | 1 | New test Case: ProSe-UE Maximum Output Power | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162880 | 2723 | 1 | New test Case: ProSe-UE Maximum Power Reduction test case | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162884 | 2599 | 1 | Update to FDD PDSCH Closed Loop Multi-Layer Spatial Multiplexing 4x2 for Dual Connectivity | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162885 | 2600 | 1 | Update to TDD PDSCH Closed Loop Multi-Layer Spatial Multiplexing 4x2 for Dual Connectivity | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162886 | 2601 | 1 | Update to sustained data rate performance for Dual Connectivity 64QAM | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162887 | 2641 | 1 | Addition of test case for FDD sustained data rate performance for Dual Connectivity 256QAM | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162888 | 2642 | 1 | Addition of test case for TDD sustained data rate performance for Dual Connectivity 256QAM | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162890 | 2728 | - | Adding Low Wgap definition to section 3 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162952 | 2632 | 1 | Update to TC 6.3.5A.2.3 Power Control Relative power tolerance for CA (intra-band non-contiguous DL CA and UL CA) | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162953 | 2525 | 1 | CR for Clause 9.3.8.2 in 36.521-1 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162954 | 2591 | 1 | New TC: TDD PDSCH Closed Loop Single-layer Spatial Multiplexing on antenna ports 7 or 8 with TM9 Interference Model - Enhanced Performance Requirement Type B | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162967 | 2624 | 2 | Addition CA combinations in intra-band contiguous DL CA and UL CA transmitter test cases | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162969 | 2660 | 1 | Update BW definition for band 66 in clause 5 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162973 | 2701 | 1 | Band 65 introduction to Chapter 5 in 36.521-1 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162974 | 2703 | 1 | Band 65 introduction to Chapter 6.6.3 in 36.521-1 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162975 | 2636 | 1 | Addition of Band 45 in Spurious emission test cases | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162976 | 2540 | 2 | 6.3.4E.1 – General ON/OFF time mask for UE category 0 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162977 | 2548 | 1 | Introduction of AMPR for inter-band DL CA and UL CA for UL 64QAM | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162978 | 2541 | 1 | 6.3.4.1 – General ON/OFF time mask. | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162979 | 2549 | 1 | Introduction of EVM for inter-band UL CA for UL 64QAM | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162980 | 2550 | 1 | Introduction of ASEM for inter-band DL CA and UL CA for UL 64QAM | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162981 | 2551 | 1 | Introduction of ACLR for inter-band DL CA and UL CA for UL 64QAM | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162982 | 2552 | 1 | Introduction of ASE for inter-band DL CA and UL CA for UL 64QAM | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162983 | 2657 | 1 | Updates of test point for A-Spur for UL 64QAM | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162985 | 2673 | 1 | New CA band combination CA_8A-40A – Updates of 5.4.2A Channel Bandwidth | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162987 | 2664 | 1 | Change of test case applicability for A-MPR for NS_05 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162989 | 2566 | 1 | Update of TC 6.6.2.2A.2 for CA_4A-5A (2DL-2UL) | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162990 | 2648 | 1 | New CA band combinations CA_28A-41A and CA_28A-41C - Updates of 7.3A Refsens | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162991 | 2716 | 1 | Introduction of new TC7.7A.7 4DL CA | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162992 | 2604 | 1 | Introduction of FDD PCFICH/PDCCH Transmit Diversity 4x4 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162993 | 2532 | 1 | Update test case 6.2.4A.2:AMPR for 2UL inter-band CA | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162995 | 2658 | 1 | Addition of a new TC for Minimum Output Power for CA (inter-band DL CA and UL CA) | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162996 | 2659 | 1 | Addition of a new TC for Minimum Output Power for CA (intra-band non-contiguous DL CA and UL CA) | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-162997 | 2682 | 1 | Introduction of test case 6.6.2.2A.3 - Additional Spectrum Emission Mask for CA (intra-band non-contiguous DL CA and UL CA) | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-163100 | 2588 | 1 | Introduction of new eMTC RF test case 6.3.5EA.1 for category M1 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-163101 | 2589 | 1 | Introduction of new eMTC RF test case 6.3.5EA.2 for category M1 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-163102 | 2705 | 1 | Addition of new test case for aggregate power control tolerance for UE category M1 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-163103 | 2706 | 1 | Addition of New test case for Maximum input level for UE category M1 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-163104 | 2707 | 1 | Addition of New test case for Adjacent Channel Selectivity (ACS) for category M1 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-163107 | 2568 | 1 | New D2D Tx test case 6.5.1D.1 Frequency error for ProSe Direct Discovery | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-163110 | 2558 | 1 | New 2x4 test case: PDSCH CLSM TypeA | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-163119 | 2708 | 1 | Introduction of new TC7.3A.9 4DL CA | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-163120 | 2709 | 1 | Introduction of new TC7.4A.7 4DL CA | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-163121 | 2711 | 1 | Introduction of new TC7.5A.7 4DL CA | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-163122 | 2712 | 1 | Introduction of new TC7.6.1A.7 4DL CA | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-163123 | 2713 | 1 | Introduction of new TC7.6.2A.7 4DL CA | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-163124 | 2715 | 1 | Introduction of new TC7.6.3A.7 4DL CA | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-163125 | 2717 | 1 | Introduction of new TC7.8.1A.7 4DL CA | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-163126 | 2718 | 1 | Updates of delta TIB,c values and table references for delta TIB,c for TC 6.2.5 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-163127 | 2727 | 1 | Introduction of CA 1A-8A-11A in chapters 5, 6 and 7 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-163129 | 2529 | 1 | Addition of new test case 7.9_1 Spurious emissions with 4 Rx antenna ports | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-163130 | 2534 | 1 | Revise Power Control Relative power tolerance for CA (intra-band contiguous DL CA and UL CA) test case 6.3.5A.2.1 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-163132 | 2652 | 1 | New CA band combinations CA_28A-42A and CA_28A-42C - Updates of 7.3A Refsens | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-163192 | 2631 | 2 | Update to TC 6.3.5A.2.2 Power Control Relative power tolerance for CA (inter-band DL CA and UL CA) | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-163194 | 2702 | 2 | Band 65 introduction to Chapter 6.2 in 36.521-1 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-163195 | 2663 | 2 | Change of test points and requirements for A-MPR for NS_05 | 13.1.0 | 13.2.0 |
| 2016-06 | RAN#72 | R5-163196 | 2726 | 1 | Correction to the UL CA Frequency Error Testcase | 13.1.0 | 13.2.0 |
| 2016-09 | RAN#73 | R5-165046 | 2742 | 2 | Introduction of new definitions for NB-IoT in Chapter 3 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165047 | 2744 | 2 | Introduction of general information for NB-IoT in Chapter 4 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165048 | 2745 | 2 | Introduction of operating bands for NB-IoT in Chapter 5 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165049 | 2730 | 2 | Introduce Downlink Fixed Reference Channels for UE category NB1 Receiver tests | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165050 | 2731 | 2 | Introduce GSM Interferer for UE category NB1 Adjacent channel selectivity tests | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165051 | 2736 | 2 | New test case: 6.2.2EA UE Maximum Output Power for UE category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165057 | 2759 | 2 | Addition of FDD MPDCCH demod test case for eMTC | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165058 | 2760 | 2 | Addition of TDD MPDCCH demod test case for eMTC | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165062 | 2757 | 2 | UE Maximum Output Power for UE category NB1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165063 | 2741 | 2 | Addition of test case 6.6.2.3F for ACLR for UE category NB1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165067 | 2749 | 2 | Addition of test case 6.7F for Transmit Intermodulation for UE category NB1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165068 | 2750 | 2 | Addition of Occupied Bandwidth test case 6.6.1F for UE category NB1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165069 | 2751 | 2 | Addition of Spectrum Emission Mask test case 6.6.2.1F for UE category NB1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165070 | 2756 | 2 | Introduction of 6.5.1F Frequency Error for UE category NB1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165071 | 2763 | 2 | Introduction of new NB-IoT RF test case 6.3.4F.1 for category NB1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165072 | 2765 | 2 | Introduction of test case 6.2.3F - Maximum Power Reduction (MPR) for UE category NB1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165073 | 2766 | 2 | Introduction of test case 6.6.3F.1 - Transmitter Spurious emission for UE category NB1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165074 | 2758 | 2 | Maximum input level for UE category NB1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165075 | 2729 | 2 | Introduce Adjacent Channel Selectivity (ACS) for UE category NB1 test case 7.5F | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165076 | 2733 | 2 | Introduce Downlink Physical Channels for UE category NB1 Receiver tests | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165077 | 2764 | 2 | Addition of the TC 6.6.2.3EA ACLR for UE category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165094 | 2776 | - | New test case: 6.5.1EA Frequency Error for UE category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165095 | 2777 | - | New test case: 6.5.2.1EA.1 Error Vector Magnitude for UE category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165096 | 2778 | - | New test case: 6.5.2.1EA.2 PUSCH-EVM with exclusion period for UE category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165097 | 2779 | - | New test case: 6.5.2.2EA Carrier leakage for UE category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165098 | 2780 | - | New test case: 6.5.2.3EA In-band emissions for non allocated RB for UE category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165099 | 2781 | - | New test case: 6.5.2.4EA EVM equalizer spectrum flatness for UE category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165100 | 2782 | - | New test case: 6.6.2.1EA Spectrum Emission Mask for UE category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165101 | 2783 | - | New test case: 6.6.2.2EA Additional Spectrum Emission Mask for UE category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165117 | 2797 | - | Changes to general clauses for UE category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165158 | 2800 | - | Introduction of ProSe RF TC 7.5D | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165159 | 2801 | - | Introduction of ProSe RF TC 7.6.3D | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165160 | 2802 | - | Introduction of ProSe RF TC 7.8.1D | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165210 | 2808 | - | New CA band combination CA_1A-40A and CA_3A-40A - Updates of 7.3A Refsens | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165225 | 2809 | - | Update minimum test time into Annex G for TC 8.9.1.1.3,8.9.1.2.3 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165228 | 2810 | - | Update to TC 9.7.2.1,9.7.2.2 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165243 | 2812 | - | Addition of new 4Rx Test Cases - CQI reporting / AWGN / rank 1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165244 | 2813 | - | Addition of new 4Rx Test Cases - CQI reporting / AWGN / rank 2 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165245 | 2814 | - | Addition of new 4Rx Test Cases - CQI reporting / AWGN / rank 4 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165247 | 2815 | - | Addition of new 4Rx Test Cases - CQI reporting / AWGN / rank 3 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165248 | 2816 | - | Uncertainties and Test Tolerances for new 4Rx Test Cases - CQI reporting / AWGN | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165250 | 2817 | - | Addition of channel matrices for 4Rx in Annex B.1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165310 | 2822 | - | Update to FDD PCFICH/PDCCH transmit diversity 2x4 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165311 | 2823 | - | Update to FDD PCFICH/PDCCH transmit diversity 4x4 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165322 | 2833 | - | Update minimum requirement of Reference sensitivity level for all Rel-13 CA configurations | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165334 | 2834 | - | Correction of test requirements of In-band emissions for non allocated RB | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165366 | 2837 | - | Update to test case 8.10.4.1.1 FDD distributed EPDCCH performance 2x4 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165368 | 2839 | - | Update to test case 8.10.4.1.2 TDD distributed EPDCCH performance 2x4 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165369 | 2840 | - | Update to test case 8.10.4.2.1 FDD localized EPDCCH performance 2x4 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165370 | 2841 | - | Introduction of a clause for Demodulation of PBCH (enhanced coverage) | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165371 | 2842 | - | Update to test case 8.10.4.2.2 TDD localized EPDCCH performance 2x4 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165395 | 2851 | - | Correction to the test requirement for General ON/OFF time mask for UE category 0 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165413 | 2854 | - | Correction for UL 64QAM test cases to TS36.521-1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165415 | 2856 | - | Update to eMTC RF test case 6.3.5EA.2 for category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165424 | 2861 | - | Update Reference sensitivity for Band 66 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165441 | 2868 | - | Introduction of EVM test for UL 64QAM + 2UL CA (intra-band non-contiguous DL CA and UL CA) | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165443 | 2870 | - | Introduction of test requirements for AMPR inter-band UL CA + UL 64QAM | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165444 | 2871 | - | Introduction of TT in Annex F for UL 64QAM + 2UL CA | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165454 | 2872 | - | Correction to test case 7.3E Reference sensitivity level for UE category 0 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165485 | 2876 | - | Removal of SCC allocation in TCs 6.5.2A.x | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165487 | 2878 | - | Annex F: Addition of Uncertainties for chapter 7 ProSe TCs | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165489 | 2880 | - | Minor correction to TC 6.6.3.3A.2 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165496 | 2885 | - | Update to Annex C for CAT M1 testing | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165510 | 2887 | - | UE Power Class test requirements in Multi-Cluster PUSCH TCs | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165512 | 2889 | - | Correction of Reference sensitivity test settings for CA_26A-41C | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165541 | 2890 | - | Introduction of additional 3DL tolerances in TC 6.2.2A.2 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165592 | 2894 | - | Adding the Diversity characteristics requirements in chapter 7.2 for category NB1 UE. | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165624 | 2909 | - | New TC: FDD PDSCH Closed Loop Single-layer Spatial Multiplexing on antenna ports 7 or 8 with TM3 interference model - Enhanced Performance Requirement Type B | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165625 | 2910 | - | New TC: TDD PDSCH Closed Loop Single-layer Spatial Multiplexing on antenna ports 7 or 8 with TM3 interference model - Enhanced Performance Requirement Type B | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165673 | 2924 | - | Correction to CA sustained data rate tests for 256QAM | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165674 | 2925 | - | Clean up on UE DL category for 256QAM DL RMC | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165675 | 2926 | - | Correction to TDD eDL-MIMO performance test case with 256QAM | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165676 | 2927 | - | Addition of test points in 6.5.1A.1 Frequency error for CA | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165680 | 2931 | - | Correction to Annex F.3 for UL64QAM test cases | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165681 | 2932 | - | Clean up on TM4 CA and TDD FDD CA performance test cases | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165682 | 2933 | - | Correction to TDD FDD CA sustained data rate performance test cases | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165683 | 2934 | - | Correction to the minimum test time for TDD FDD CA performance test cases | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165684 | 2935 | - | Correction to TDD FDD soft buffer for 2DL CA | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165690 | 2938 | - | Correction of the test requirement for TC 6.6.2.1A.1 SEM for CA (intra-band contiguous DL CA and UL CA) | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165695 | 2942 | - | Addition of the FDD-FDD-TDD 3DL case into TC 9.6.1.3.2 TDD FDD CQI Reporting under AWGN conditions – PUCCH 1-0 for FDD PCell (3DL CA) | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165701 | 2948 | - | Change of test points and requirements for NS_05 A-MPR for UL64QAM | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165702 | 2949 | - | Change of test points and requirements for NS_05 Additional spurious emissions for UL64QAM | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165732 | 2951 | - | Correction to spurious emission band UE co-existence limits references for Rel-13 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165734 | 2952 | - | NS_24 and NS_25 A-MPR test configuration and requirements | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165735 | 2953 | - | NS_24 and NS_25 A-Spurious emission test configuration and requirements | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165785 | 2955 | - | Addition of CQI report config IE for 4Rx TM9 test cases | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165787 | 2956 | - | Addition TC 6.3.2F Minimum Output power for UE category NB1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165790 | 2957 | - | Addition TC 7.3F.1 Reference sensitivity test for UE category NB1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165820 | 2966 | - | 36.521-1 UL CA frequency error clarification | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165837 | 2970 | - | Addition Out of-band blocking with 4 Rx antenna ports TC 7.6.2_1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165838 | 2971 | - | Addition Narrow band blocking with 4 Rx antenna ports TC 7.6.3_1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165857 | 2975 | - | New test case: FDD Multiple timing reference test for ProSe direct discovery | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-165858 | 2976 | - | New test case: FDD power imbalance performance with two links for ProSe Direct communication | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166001 | 2869 | 1 | Introduction of test configurations for ASEM inter-band UL CA + UL 64QAM | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166002 | 2930 | 1 | Correction to interferer frequencies in In-band blocking for CA | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166003 | 2884 | 1 | Addition of test points to UL 64 QAM TC | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166004 | 2905 | 1 | Correction to TC 6.2.4A.1 A-MPR for CA (intra-band contiguous DL CA and UL CA) | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166005 | 2929 | 1 | Correction to Max Power for CA | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166006 | 2928 | 1 | Clean up on chapter 3, 6 and 7 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166007 | 2941 | 1 | Change of test points and requirements for NS_05 A-MPR | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166008 | 2943 | 1 | Change of test points and requirements for NS_05 Additional spurious emissions | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166009 | 2945 | 1 | Change of test points and requirements for NS_05 A-MPR for UL-MIMO | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166010 | 2947 | 1 | Change of test points and requirements for NS_05 Additional spurious emissions for UL-MIMO | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166011 | 2818 | 1 | Change of names of 3DL TCs | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166012 | 2860 | 1 | Update Reference sensitivity for additional requirement | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166013 | 2883 | 1 | Correction to applicability of Power Class 3 only UL TCs | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166022 | 2799 | 1 | Correction to Max Input power for 256QAM | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166023 | 2939 | 1 | Update of the test points of the TC 8.7.1.1_H.3 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166027 | 2771 | 1 | Update of Delta Tib tables in clause 6.2.5.3 for all Rel-13 CA configurations | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166028 | 2772 | 1 | Update of Delta Rib tables in clause 7.3.3 for all Rel-13 CA configurations | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166029 | 2773 | 1 | Update of CA operating bands and bandwidth for all Rel-13 CA configurations | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166030 | 2768 | 1 | Addition of CA_3A-5A BW set 3 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166031 | 2819 | 1 | Introduction of CA_8A-42A (2DL) to test case 7.3A.3 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166033 | 2820 | 1 | Introduction of CA_8A-42C (3DL) to test case 7.3A.5 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166034 | 2853 | 1 | Addition of CA_2A-28A in TS 36.521-1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166035 | 2858 | 1 | Update power control and in-band emission requirement for CA_8B | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166036 | 2862 | 1 | Update TC 6.6.2.1A.1 requirement for CA_8B | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166037 | 2902 | 1 | Rel 13 CA_5B RF Test Cases Section 7 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166038 | 2903 | 1 | Rel 13 CA_3C-5A RF Test Cases Section 7 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166039 | 2916 | 1 | Introduction of New TC8.7.1.1_A.4 4DL CA | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166040 | 2917 | 1 | Introduction of New TC9.6.1.1_A.3 4DL CA | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166041 | 2944 | 1 | Updates of Rx test cases for 4DL CA | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166042 | 2958 | 1 | Addition CA_2A-4A-5A-30A in TS 36.521-1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166043 | 2960 | 1 | Addition CA_2A-2A-4A-4A in TS 36.521-1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166044 | 2920 | 1 | Update TC 6.6.2.3A.1 and 6.6.2.3A.1_1 requirement for CA_8B | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166045 | 2921 | 1 | Update TC 6.6.3.1A.1 requirement for CA_8B | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166046 | 2922 | 1 | Update TC 6.6.3.2A.1 requirement for CA_8B | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166047 | 2923 | 1 | Update TC 6.7A.1 requirement for CA_8B | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166048 | 2940 | 1 | Addition of the FDD-FDD-TDD 3DL case into TC 8.7.5.1.2 TDD FDD CA Sustained data rate performance for FDD PCell (3DL CA) | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166060 | 2844 | 1 | New test case: 6.3.5F.1 Power Control Absolute power tolerance for UE category NB1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166061 | 2845 | 1 | New test case: 6.3.5F.2 Power Control Relative power tolerance for UE category NB1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166062 | 2846 | 1 | New test case: 7.7F Spurious response for UE category NB1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166063 | 2847 | 1 | New test case: 7.8.1F Wide band Intermodulation for UE category NB1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166064 | 2864 | 1 | Introduction of TT for Configured UE transmitted output power in Annex F | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166065 | 2865 | 1 | Resubmission of 6.2.5F Configured UE transmitted Output Power for UE category NB1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166066 | 2866 | 1 | Resubmission of 6.5.2.1F.1 Error Vector Magnitude (EVM) for UE category NB1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166067 | 2867 | 1 | Resubmission of 6.5.2.2F Carrier leakage for UE category NB1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166068 | 2895 | 1 | Addition of In-band emissions for non allocated RB test case 6.5.2.3F for UE category NB1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166069 | 2775 | 1 | New test case: 6.2.4EA Additional Maximum Power Reduction (A-MPR) for UE category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166070 | 2791 | 1 | New test case: 6.2.3EA Maximum Power Reduction (MPR) for UE category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166071 | 2792 | 1 | New test case: 6.6.3EA.1 Transmitter Spurious emissions for UE category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166072 | 2793 | 1 | New test case: 6.6.3EA.2 Spurious emission band UE co-existence for UE category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166073 | 2794 | 1 | New test case: 6.6.3EA.3 Additional spurious emissions for UE category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166074 | 2795 | 1 | New test case: 7.3EA Reference sensitivity level for UE category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166075 | 2835 | 1 | Introduction of General ON/OFF time mask for UE category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166076 | 2836 | 1 | Introduction of PRACH and SRS ON/OFF time mask for UE category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166077 | 2838 | 1 | Introduction of Occupied bandwidth for UE category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166078 | 2848 | 1 | New test case: 7.7EA Spurious response for UE category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166080 | 2850 | 1 | New test case: 7.9EA Spurious emissions UE category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166084 | 2913 | 1 | Update of TC6.3.5EA.3 Aggregate power control tolerance for UE category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166085 | 2914 | 1 | Update of TC7.4EA Maximum input level for UE category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166086 | 2915 | 1 | Update of TC7.5EA Adjacent Channel Selectivity (ACS) for UE category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166087 | 2972 | 1 | Adding narrowband and narrowband index definitions to 36.521-1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166103 | 2805 | 1 | Corrections to LC MTC test case 7.4E and 7.5E | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166104 | 2912 | 1 | Update of TC 9.7.1.2 TDD CQI reporting definition under AWGN conditions for UE category 0 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166105 | 2803 | 1 | Introduction of NAICS RF TC 8.3.1.1.7 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166106 | 2804 | 1 | Introduction of NAICS RF TC 8.3.2.1.8 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166108 | 2875 | 1 | Removal of DL allocation from UL CA TCs | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166109 | 2964 | 1 | 36.521-1: UL CA spurious updates | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166111 | 2965 | 1 | 36.521-1: A-MPR and A-SEM corrections for inter-band UL CA | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166112 | 2859 | 1 | Additional Operation Band 46 for LAA | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166113 | 2897 | 1 | Update test case 7.6.3A.3 to add LAA band combinations | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166114 | 2824 | 1 | Update to TDD PCFICH/PDCCH single-antenna performance 1x4 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166115 | 2825 | 1 | Update to TDD PCFICH/PDCCH transmit diversity 2x4 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166116 | 2826 | 1 | Update to TDD PCFICH/PDCCH transmit diversity 4x4 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166117 | 2827 | 1 | Introduction of FDD PHICH Single-antenna Port Performance 1x4 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166118 | 2828 | 1 | Introduction of FDD PHICH Transmit Diversity 2x4 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166119 | 2829 | 1 | Introduction of FDD PHICH Transmit Diversity 4x4 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166120 | 2830 | 1 | Introduction of TDD PHICH Single-antenna Port Performance 1x4 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166121 | 2831 | 1 | Introduction of TDD PHICH Transmit Diversity 2x4 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166122 | 2832 | 1 | Introduction of TDD PHICH Transmit Diversity 4x4 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166123 | 2978 | 1 | Addition of the 4Rx test case into the chapter 7 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166133 | 2980 | - | Change of test points and requirements for NS_05 A-MPR for Band 65 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166134 | 2981 | - | Change of test points and requirements for NS_05 Additional spurious emissions for Band 65 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166135 | 2982 | - | Change of test points and requirements for NS_05 A-MPR for UL-MIMO for Band 65 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166136 | 2983 | - | Change of test points and requirements for NS_05 Additional spurious emissions for UL-MIMO for Band 65 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166137 | 2950 | 1 | Correction to TC 6.6.3.2 Band 66 UE E-UTRA release requirements | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166138 | 2967 | 1 | 36.521-1 Band 66 Receiver test cases updates | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166139 | 2977 | 1 | Updates to 4Rx Demod test cases | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166141 | 2959 | 1 | D2D New test cases: Power imbalance performance with two Sidelinks | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166143 | 2901 | 1 | Rel 13 CA_5A-5A RF Test Cases Section 7 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166145 | 2937 | 1 | Addition of some 4DL CA combinations into TC 7.3A.9.5 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166147 | 2852 | 1 | Updates of 7.3A.5 Channel bandwidth for CA_1A-3C | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166148 | 2898 | 1 | Introduction to new test case 6.2.5EA Configured transmitted power for UE category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166152 | 2879 | 1 | Corrections to TC 7.9A | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166154 | 2896 | 1 | Update test case 7.6.1A.3 to add LAA band combinations | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166155 | 2877 | 1 | Correction to 256QAM CQI TCs | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166156 | 2806 | 1 | Addition of CA_4A-12A Bandwidth combination set5 information | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166157 | 2857 | 1 | Update Maximum Output Power and MPR requirement for CA_8B | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166158 | 2918 | 1 | Update of Test Cases for 4DL CA | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166160 | 2821 | 1 | Update to FDD PCFICH/PDCCH single-antenna performance 1x4 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166165 | 2855 | 1 | Update to eMTC RF test case 6.3.5EA.1 for category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166166 | 2900 | 1 | Introduction to new test case 6.3.3EA UE Transmit OFF power for UE category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166169 | 2873 | 1 | Corrections to TC 7.3A.4 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166171 | 2899 | 2 | Introduction to new test case 6.3.2EA Minimum Output Power for UE category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166303 | 2874 | 2 | Corrections to TC 7.3A.5 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166309 | 2789 | 2 | New test case: 9.8.1.1 FDD and half-duplex FDD CQI reporting definition under AWGN conditions in enhanced coverage | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166310 | 2796 | 2 | New test case: 8.11.1.1.1 FDD and half-duplex FDD (Fixed Reference Channel) demodulation of PDCCH in enhanced coverage | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166311 | 2811 | 1 | Adding RMCs for CAT M1 testing to Annex A | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166312 | 2849 | 2 | New test case: 7.8.1EA Wide band intermodulation for UE category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166313 | 2891 | 2 | Introduction of new test case In-band blocking for UE category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166314 | 2892 | 2 | Introduction of new test case Out-of-band blocking for UE category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166315 | 2893 | 2 | Introduction of new test case Narrow band blocking for UE category M1 | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166319 | 2973 | 2 | 36.521-1 Correction of intra-band cont CA spurious co-existence test case for Rel-10 and Rel-11 UEs | 13.2.0 | 13.3.0 |
| 2016-09 | RAN#73 | R5-166053 | 2904 | 1 | New CA band combination CA_1A-41A-42A - Updates of 5.4.2A Channel bandwidth | 13.3.0 | 14.0.0 |
| 2016-09 | RAN#73 | R5-166054 | 2906 | 1 | New CA band combination CA_1A-41A-42A - Updates of 6.2.5.3 Delta TIB | 13.3.0 | 14.0.0 |
| 2016-09 | RAN#73 | R5-166055 | 2907 | 1 | New CA band combination CA_1A-41A-42A - Updates of 7.3.3 Delta RIB | 13.3.0 | 14.0.0 |
| 2016-09 | RAN#73 | R5-166056 | 2908 | 1 | New CA band combination CA_1A-41A-42A - Updates of 7.3A Refsens | 13.3.0 | 14.0.0 |
| 2016-12 | RAN#74 | R5-168019 | 2984 | - | Adding MPDCCH aggregation level to Annex C for CAT M1 testing | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168042 | 2995 | - | Introduction of numbering rules guideline for category M1 in Annex I | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168123 | 3007 | - | Completion of 4Rx Test Cases for CQI reporting under AWGN | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168125 | 3009 | - | Correction to Initial conditions of TC 6.2.4A.2_1 for NS_03/NS_01 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168126 | 3010 | - | Correction to Initial conditions of TC 6.6.2.2A.2_1 for NS_03/NS_01 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168189 | 3014 | - | Chapter 3 introduction of symbols for NB-IoT | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168190 | 3015 | - | Chapter 5 change of frequency bands and channel arrangement for NB-IoT | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168191 | 3016 | - | 6.2.5F Configured UE transmitted Output Power for UE category NB1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168194 | 3019 | - | Annex A Introduction of UL RMC for NB-IoT | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168195 | 3020 | - | Introduction of TT for EVM and Carrier leakage test cases | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168209 | 3021 | - | Derivation of Test Requirements for 7.5A.4 ACS for intra-band non-contiguous DL CA | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168294 | 3042 | - | Update to eMTC RF test case 6.3.5EA.1 and 6.3.5EA.2 for category M1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168310 | 3047 | - | Updated chapter 2, 3, 4 and 5 for ProSe | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168311 | 3048 | - | Updated of Delta Tib tables in clause 6.2.5.3 for all Rel-14 CA configurations | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168314 | 3051 | - | Correction to RF TC 6.6.3.1A.1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168368 | 3053 | - | Updates to section 7 4Rx single carrier TCs | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168400 | 3063 | - | Band 70 introduction to 36.521-1 Chapter 5 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168401 | 3064 | - | Band 70 introduction to 36.521-1 output power requirements | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168405 | 3066 | - | Correction to TX-RX frequency separation for Band 65 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168420 | 3069 | - | Update of TC 6.6.3.1A.2 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168478 | 3070 | - | Introduction of Power Class 2 in power control for HPUE | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168484 | 3071 | - | Update to 4 test cases 8.10.4.1.1, 8.10.4.1.2, 8.10.4.2.1 and 8.10.4.2.2 by adding connection diagram | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168555 | 3093 | - | Updates of the test number in TC 8.7.5.1_H.2 TDD FDD CA SDR for FDD PCell (3DL CA) for 256QAM in DL | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168557 | 3094 | - | Correction of the test point for TC 7.3A.9 REFSENS 4DL CA | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168595 | 3103 | - | Update to In-band blocking requirements 7.6.1A.0 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168604 | 3105 | - | Clean-up of 7.3A.3 test configuration tables | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168647 | 3115 | - | Correction of RB allocation in Additional Maximum Power Reduction (A-MPR) for CA and Additional Spectrum Emission Mask for CA | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168654 | 3120 | - | Corrections of Spurious Emissions TC for CA | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168668 | 3126 | - | Correction to tolerance value for CA_NS_01 for CA A-MPR test 6.2.4A.1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168669 | 3127 | - | Correction to UL RB allocation in 6.5.1A Frequency error for CA | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168670 | 3128 | - | Clarification of test requirement for CA MPR test cases 6.2.3A.1 and 6.2.3A.1_1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168671 | 3129 | - | Correction of Highest NRB_agg and REFSENS in 7.3A.5 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168672 | 3130 | - | Correction to DL modulation parameter for CA 5A-5A in Max Input Level test | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168675 | 3133 | - | Correction to reference channel of Multi-Layer Spatial Multiplexing test case for CA | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168676 | 3134 | - | Corrections regarding power allocation for TM9 performance and CQI tests | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168749 | 3151 | - | Update minimum conformance requirements for receiver 4 Rx antenna ports test cases and legacy test cases | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168785 | 3167 | - | RF: Correction to RMC R.31-6 FDD | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168792 | 3171 | - | Updates to test case 6.2.3EA - MPR for category M1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168794 | 3172 | - | Updates to test case 6.2.4EA - A-MPR for category M1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168795 | 3173 | - | Updates to test case 6.6.2.1EA - SEM for category M1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168796 | 3174 | - | Updates to test case 6.6.2.2EA – Additional SEM for category M1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168797 | 3175 | - | Updates to test case 6.6.3EA.3 – Additional Spurious emissions for category M1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168803 | 3179 | - | Introduction of new test case 7.6.1F In-band blocking for UE category NB1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168805 | 3181 | - | Introduction of new test case 7.6.2F Out-of-band blocking for UE category NB1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168807 | 3182 | - | CSI-RS-Config-r10 message content aligned to minimum conformance requirements for TC 8.3.1.1.2_D | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168816 | 3184 | - | NB-IoT test frequencies algorithm for TRx test cases | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168827 | 3188 | - | Addition Reference sensitivity with repetitions test for UE category NB1 7.3F.2 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168829 | 3189 | - | Default operation mode definition for TRx TCs | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168845 | 3191 | - | CA_70C introduction to 36.521-1 receiver requirements | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168847 | 3192 | - | Update UL RMC for CA_1A-3A-7A in 7.3A.5 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168849 | 3193 | - | Update test repetitions tables for receiver 4CC test cases | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168851 | 3194 | - | Update test case 6.5.2A.2.1 to remove editor’s notes | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168852 | 3195 | - | Update reference sensitivity level for some CA Configurations in test 7.3A.6 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168857 | 3197 | - | Editorial correction in TC 6.2.4A.1_1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168858 | 3198 | - | Update reference tables in ASEM, ACLR, ASE for intra-band contiguous UL CA for UL 64QAM | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168859 | 3199 | - | Update test cases 6.2.4A.1 and 6.6.2.2A.1 to remove editor’s notes | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168974 | 3210 | - | Addition of TM10 CQI reporting test cases with NAICS | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168975 | 3211 | - | Test tolerance update to MPDCCH performance test cases | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168979 | 3213 | - | Update of test tolerance of 4-Rx Type A demod test cases | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168981 | 3215 | - | Test Tolerances for FDD PDSCH Closed Loop Single Layer Spatial Multiplexing on antenna ports 7 or 8 with TM9 Interference Model – Enhanced Performance Requirement Type B (8.3.1.1.4) | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168982 | 3216 | - | Test Tolerances for FDD PDSCH Closed Loop Single-layer Spatial Multiplexing on antenna ports 7 or 8 with TM3 interference model - Enhanced Performance Requirement Type B (8.3.1.1.6) | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168983 | 3217 | - | Test Tolerances for TDD PDSCH Closed Loop Single-layer Spatial Multiplexing on antenna ports 7 or 8 with TM9 Interference Model - Enhanced Performance Requirement Type B (8.3.2.1.5) | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168984 | 3218 | - | Test Tolerances for TDD PDSCH Closed Loop Single-layer Spatial Multiplexing on antenna ports 7 or 8 with TM3 Interference Model - Enhanced Performance Requirement Type B (8.3.2.1.7) | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-168985 | 3219 | - | Test Tolerances for FDD and TDD CQI Reporting under fading conditions - PUCCH 1-1 (CSI Reference Symbol) TM9 - Enhanced Receiver Type B (9.3.8.2.1 and 9.3.8.2.2) | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169045 | 3229 | - | Addition Transmit intermodulation for UE category M1 test case 6.7EA | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169502 | 3080 | 1 | Update to test case 8.7.5.1.2 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169506 | 3006 | 1 | Update of B3/39 UE co-existence requirements | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169507 | 3058 | 1 | Updates to SEM Test Cases for HPUE | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169508 | 3067 | 1 | Correction to Band 66 UL-MIMO test requirements 6.2.3B, 6.2.4B | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169509 | 3068 | 1 | Correction to NS_24 and NS_25 TC 6.6.3.3 test requirements | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169510 | 3108 | 1 | Correction of test description for NS_05 A-MPR | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169511 | 3109 | 1 | Correction of test description for NS_05 Additional spurious emissions | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169512 | 3138 | 1 | Correction of power tolerance in A-MPR TC | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169513 | 3196 | 1 | Update TC 6.6.3.3A.1 to remove editor’s notes and modify additional requirements for CA_NS_07 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169514 | 3097 | 1 | Addition of missing CA_7A-8A for Refsens test | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169519 | 3043 | 1 | Introduction of new NB-IoT RF test case 6.3.3F for category NB1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169520 | 3212 | 1 | Addition of demod test case for NPDSCH with In-band mode | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169521 | 3074 | 1 | Update to test case 6.3.5F.2 Power Control Relative power tolerance for UE category NB1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169522 | 3076 | 1 | Update to test case 7.8.1F Wide band Intermodulation UE category NB1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169523 | 3214 | 1 | Update to Annex H | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169524 | 3099 | 1 | Supplementation of test case 6.7F for Transmit Intermodulation for UE category NB1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169525 | 3177 | 1 | Updates in test case 6.6.3F.1, Transmitter Spurious emissions for UE category NB1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169526 | 3185 | 1 | Corrections for NB-IoT Rx test cases in TS 36.521-1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169527 | 3186 | 1 | Corrections for NB-IoT Tx test cases in TS 36.521-1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169528 | 3227 | 1 | Addition of demod test case for NPDCCH with In-band mode | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169531 | 3056 | 1 | Corrections to TC 7.3A.9 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169532 | 3078 | 1 | Add New TC of 9.6.1.3.3 for TDD FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for FDD PCell (4DL CA) | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169533 | 3079 | 1 | Add New TC of 9.6.1.4.3 for TDD FDD CQI Reporting under AWGN conditions – PUCCH 1-0 for TDD Pcell | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169535 | 3089 | 1 | Addition of the new TC 8.7.5.1.3 TDD FDD CA Sustained data rate performance for FDD PCell (4DL CA) | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169536 | 3091 | 1 | Addition of the new TC 8.2.3.1.1.3 TDD FDD CA PDSCH Single Antenna Port Performance for FDD PCell (4DL CA) | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169537 | 3092 | 1 | Addition of the new TC 8.7.5.1_H.3 TDD FDD CA Sustained data rate performance for FDD PCell (4DL CA) for 256QAM in DL | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169539 | 3111 | 1 | New CA band combination CA_1A-11A-18A - Updates of 7.3A Refsens | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169541 | 3112 | 1 | New CA band combination CA_41C-42C - Updates of 7.3A Refsens | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169543 | 3113 | 1 | New CA band combinations CA_1A-18A, 1A-26A, 1A-28A, 1A-42A and 18A-28A - Updates of 6.2.2A UE maximum output power | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169544 | 3117 | 1 | New CA band combinations CA_1A-18A, 1A-26A, 1A-28A, 1A-42A and 18A-28A - Updates of 6.6.3.1A Transmitter Spurious emissions | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169545 | 3118 | 1 | New CA band combinations CA_1A-18A, 1A-26A, 1A-28A, 1A-42A and 18A-28A - Updates of 6.6.3.2A Spurious emission band UE co-existence | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169546 | 3137 | 1 | Introduction of exception for CA_4A-12A-30A in Refsense TC | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169550 | 3164 | 1 | Correction of CA_42-42 sub-block CA configuration | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169567 | 2994 | 1 | Updates of 7.3A Refsens for CA_1A-3A-41A | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169568 | 3049 | 1 | Updated of Delta Rib tables in clause 7.3.3 for all Rel-14 CA configurations | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169569 | 3050 | 1 | Updated of Reference sensitivity level in clause 7.3A.0 for all Rel-14 CA configurations | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169570 | 3086 | 1 | Updates of the test point of the TC 6.6.2.3EA ACLR for UE category M1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169571 | 3125 | 1 | Correction to Rx test cases for Cat M1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169572 | 3228 | 1 | Update of CA operating bands and bandwidth for all CA configurations as per TS 36.101 v14.1.0 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169573 | 3180 | 1 | Introduction of FDD and Half-Duplex FDD Demodulation of MPDCCH in CE Mode B Test Case (not implementable) | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169574 | 3183 | 1 | Introduction of TDD Demodulation of MPDCCH in CE Mode B Test Case (not implementable) | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169610 | 3170 | 1 | Introduction of NAICS demod test cases 8.3.1.1.5 (FDD) and 8.3.2.1.6 (TDD) | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169612 | 3022 | 1 | Introduction of ProSe Transmit Off power Test case 6.3.3D.1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169613 | 3060 | 1 | Updates to ProSe Receiver TCs | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169615 | 3013 | 1 | Update of TC 6.6.3.2A.2 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169616 | 3055 | 1 | Corrections to TC 7.3A.6 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169620 | 3008 | 1 | update Annex A with 4Rx RMCs | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169621 | 3011 | 1 | Addition of new 4Rx Test Case – TDD PMI reporting | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169622 | 3012 | 1 | Addition of new 4Rx Test Case - TDD FDD CA SDR (2DL CA) | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169623 | 3023 | 1 | Introduction of 4Rx new test case “FDD sustained data rate performance 4X4” | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169624 | 3024 | 1 | Introduction of 4Rx new test case “TDD sustained data rate performance 4X4” | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169625 | 3025 | 1 | Introduction of new 4Rx test case 9.9.2.1.1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169626 | 3026 | 1 | Introduction of new 4Rx test case 9.9.2.1.2 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169627 | 3027 | 1 | Introduction of new 4Rx test case 9.9.2.2.1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169628 | 3028 | 1 | Introduction of new 4Rx test case 9.9.2.2.2 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169629 | 3030 | 1 | Update connection diagram for PCFICH/PDCCH performance with 4Rx antenna ports | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169630 | 3031 | 1 | Update to FDD PHICH Single-antenna Port Performance 1x4 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169631 | 3032 | 1 | Update to FDD PHICH Transmit Diversity 2x4 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169632 | 3033 | 1 | Update to FDD PHICH Transmit Diversity 4x4 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169633 | 3034 | 1 | Update to TDD PHICH Single-antenna Port Performance 1x4 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169634 | 3035 | 1 | Update to TDD PHICH Transmit Diversity 2x4 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169635 | 3036 | 1 | Update to TDD PHICH Transmit Diversity 4x4 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169636 | 3046 | 1 | Update Reference sensitivity for additional requirement and TC7.3_1 test requirement | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169637 | 3054 | 1 | Updates to 4 Layer Spatial Multiplexing TCs | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169638 | 3110 | 1 | Correction of FDD and TDD Open-loop spatial multiplexing TCs with 3 Layer Multiplexing with 4 Tx Antenna Ports | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169639 | 3123 | 1 | Correction to 4Rx demodulation tests | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169640 | 3178 | 1 | Introduction of FDD 4Rx PDSCH demodulation test cases | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169650 | 3065 | 1 | Band 70 introduction to 36.521-1 receiver requirements | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169652 | 3156 | 1 | Introduction of Band 41 HPUE Maximum Output Power | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169653 | 3157 | 1 | Introduction of Band 41 HPUE Maximum Output Power for CA | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169654 | 3158 | 1 | Introduction of Band 41 HPUE Maximum Output Power for UL-MIMO | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169655 | 3159 | 1 | Introduction of Band 41 HPUE Maximum Power Reduction | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169656 | 3168 | 1 | Introduction of Power Class 2 in test case 6.2.5_1 Configured UE transmitted Output Power for HPUE | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169657 | 3169 | 1 | Addition of TDD RMC for UL-DL configuration 0 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169659 | 3136 | 1 | Correction of CQI Reporting under AWGN conditions - PUCCH 1-1 (Cell-Specific Reference Symbols) TC | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169660 | 3059 | 1 | Updates to SEM Test Case for category NB1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169662 | 3106 | 1 | Correction to EPDCCH SDR tests | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169665 | 3122 | 1 | Corrections to 256QAM sustained data rate performance test cases | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169666 | 3082 | 1 | Introduction of Rel-13 CA new test case “TDD Sustained data rate performance for CA (4DL CA)” | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169667 | 3114 | 1 | New CA band combinations CA_1A-18A, 1A-26A, 1A-28A, 1A-42A and 18A-28A - Updates of 6.2.4A Additional Maximum Power Reduction (A-MPR) | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169668 | 3116 | 1 | New CA band combinations CA_1A-18A, 1A-26A, 1A-28A, 1A-42A and 18A-28A - Updates of 6.6.2.2A Additional Spectrum Emission Mask | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169669 | 3119 | 1 | New CA band combinations CA_1A-18A, 1A-26A, 1A-28A, 1A-42A and 18A-28A - Updates of 6.6.3.3A Additional spurious emissions | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169671 | 3220 | 1 | Addition of CA band combinations into TC 7.3A.9 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169672 | 3221 | 1 | Update of 4DL CA RX Test Cases | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169675 | 3203 | 1 | Addition of new case FDD RI Reporting- PUCCH 1-1 4x4 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169676 | 3204 | 1 | Addition of new case TDD RI Reporting- PUSCH 3-1 4x4 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169677 | 3206 | 1 | Addition of new case FDD RI Reporting- PUCCH 1-1 for eDL-MIMO 1x4 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169678 | 3207 | 1 | Addition of new case TDD RI Reporting- PUCCH 1-1 for eDL-MIMO 1x4 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169681 | 3160 | 1 | Update some receiver test cases to add LAA band combinations | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169682 | 3202 | 1 | Update test case 7.8.1A.3 to add LAA band combinations | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169683 | 3044 | 1 | Updated to RF test case 6.3.4F.1 for Category NB1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169684 | 3045 | 1 | Introduction of new NB-IoT RF test case 6.3.4F.2 for category NB1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169686 | 3075 | 1 | Update to test case 7.7F Spurious response for UE category NB1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169687 | 3088 | 1 | Updates of the TC 6.6.3F.2 Spurious emission band UE co-existence for category NB1 UE | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169688 | 3098 | 1 | Supplementation of test case 6.6.2.3F for ACLR for UE category NB1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169689 | 3100 | 1 | Update of test case 6.6.1F Occupied bandwidth for UE category NB1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169690 | 3101 | 1 | Update of test case 6.6.2.1F Spectrum Emission Mask for UE category NB1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169691 | 3102 | 1 | Update of test case 6.5.2.3F In-band emissions for non allocated RB for UE category NB1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169692 | 3124 | 1 | Update Adjacent Channel Selectivity (ACS) for UE category NB1 test case 7.5F | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169693 | 3176 | 1 | Updates in test case 6.2.3F, Maximum Power Reduction (MPR) for UE category NB1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169694 | 3225 | 1 | Update of Test Case 6.5.1F Frequency Error for UE category NB1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169696 | 3017 | 1 | 6.5.2.1F.1 Error Vector Magnitude (EVM) for NB-IoT | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169697 | 3018 | 1 | 6.5.2.2F Carrier leakage for UE category NB1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169699 | 2996 | 1 | Updates to HD-FDD scheduling pattern for cat M1 RMCs in annex A | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169700 | 2998 | 1 | Update test case: 8.11.1.1.1, FDD Closed-loop spatial multiplexing performance for UE category M1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169701 | 2999 | 1 | New test case: 8.11.1.1.2, FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category M1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169702 | 3000 | 1 | New test cases: 8.11.1.1.3, FDD PDSCH Transmit Diversity 2x1 for UE category M1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169703 | 3001 | 1 | New test case: 8.11.1.2.1, TDD Closed-loop spatial multiplexing performance for UE category M1 (Cell-Specific Reference Symbols) | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169704 | 3002 | 1 | New test case: 8.11.1.2.2, TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category M1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169705 | 3003 | 1 | New test case: 8.11.1.2.3, TDD PDSCH Transmit Diversity for UE category M1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169706 | 3005 | 1 | Updating eMTC test case 9.8.1.1, FDD and Half duplex FDD CQI reporting definition under AWGN conditions for UE category M1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169707 | 3004 | 1 | New test case: 8.11.1.2.3_1, TDD PDSCH Transmit Diversity for UE category M1 (CEModeB) | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169713 | 3131 | 1 | Correction to PCC SCC frequency mapping for CA_66A-66A | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169715 | 3132 | 1 | Correction of UL RMC for sustained data rate CA test cases | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169716 | 3162 | 1 | Update test procedure 9.2.1.5_E.1 and 9.2.1.6_E.1 test cases to align it with minimum conformance requirements | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169719 | 3081 | 1 | Introduction of Rel-13 CA new test case “FDD Sustained data rate performance for CA (5DL CA)” | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169720 | 3083 | 1 | Introduction of Rel-13 CA new test case “FDD CQI Reporting under AWGN conditions – PUCCH 1-0 for CA (5DL CA)” | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169721 | 3104 | 1 | Introduction of new TC 8.2.2.1.1_A.3 4DL CA | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169722 | 3222 | 1 | New TC: FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for 5DL CA | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169723 | 3223 | 1 | New TCs: TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for 4DL CA | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169730 | 3121 | 2 | Correction to test points for test cases refer to 6.2.3A | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169732 | 3073 | 2 | Update to test case 6.3.5F.1 Power Control Absolute power tolerance for UE category NB1 | 14.0.0 | 14.1.0 |
| 2016-12 | RAN#74 | R5-169735 | 3166 | 2 | Update test requirements for test case 6.6.3.2A.1 | 14.0.0 | 14.1.0 |
| 2017-03 | RAN#75 | R5-170604 | 3238 | - | Update 4Rx test cases 9.9.4.1.1/9.9.4.1.2/9.9.4.2.1/9.9.4.2.2 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170606 | 3239 | - | Update 4Rx test cases 9.9.4.1.1/9.9.4.1.2/9.9.4.2.1/9.9.4.2.2 information in Annex F and G. | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170627 | 3249 | - | Resubmission of R5-170017 Update of editor's note in NB-IoT receiver RF tests related to scheduling pattern issue | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170630 | 3250 | - | Resubmission of R5-170073 Update of editor's note in NB-IoT transmitter RF tests related to scheduling pattern issue | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170631 | 3251 | - | Resubmission of R5-170074 Update of test parameters for NPDCCH tests | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170632 | 3252 | - | Resubmission of R5-170328 New TC 6.3.5F.3 Aggregate power control tolerance for category NB1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170633 | 3253 | - | Resubmission of R5-170333 Introduction of RMCs for NB-IoT demodulation tests | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170641 | 3258 | - | Update of TC 6.6.2.3_1 Adjacent Channel Leakage power Ratio for HPUE | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170694 | 3269 | - | Updates of the CQI reporting TC 9.6.1.4.2 for TDD FDD CA | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170750 | 3273 | - | Correction of typos in test cases 6.3.5.1, 6.3.5A.1.1, 6.3.5A.1.2, 6.3.5A.1.3, 6.3.5B.1, 6.3.5E.1 and 6.3.5EA.1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170751 | 3274 | - | Update to test cases 6.5.1D.1 and 6.5.1D.2 Frequency error for ProSe | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170757 | 3276 | - | Uncertainties and Test Tolerances update to FDD and TDD CQI Reporting under fading conditions - PUCCH 1-0 - Enhanced Performance Requirement Type A 1x4 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170758 | 3277 | - | Uncertainties and test tolerances update to FDD and TDD CQI Reporting under fading conditions - PUCCH 1-1 - Enhanced Performance Requirement Type A 2x4 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170760 | 3278 | - | Update of test cases 8.7.9 and 8.7.10 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170762 | 3279 | - | Update of test cases 8.7.1.1_A.5, 8.7.2.1_A.3 and 9.6.1.1_A.4 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170774 | 3280 | - | Beamforming model missing in chapter 9 TM9 receiver type A tests | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170806 | 3287 | - | Correction of number of EREGs per ECCE for special subframe mentioned in TC 8.7.4 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170811 | 3291 | - | Correction of description of TC 8.2.2.4.2_1 FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 (Release 9 and Forward) | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170813 | 3292 | - | Introduction of Minimum Test time for R.73 TDD | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170814 | 3293 | - | Update of Mapping of CQI Index to Modulation Coding Scheme | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170816 | 3295 | - | Correction of release of TC Error Vector Magnitude (EVM) for CA (inter-band DL CA and UL CA) for UL 64QAM | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170823 | 3296 | - | Removal of the power class restriction in the test applicability for Test Cases 6.2.2A.1 and 6.2.2A.2 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170894 | 3304 | - | Correction to feICIC CQI TCs | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170895 | 3305 | - | Annex E: Updates for NB-IoT | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170896 | 3306 | - | Annex E: Update to averaged EVM for D2D | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170897 | 3307 | - | Update of requirements in TC 6.2.2A.2 for CA_2A-5A and CA_2A-12A | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170905 | 3310 | - | Re-addition of Band 41 HPUE Maximum Output Power | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170908 | 3312 | - | Update of TC 6.6.3.3A.2 for CA_2A-5A and CA_2A-12A | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170909 | 3313 | - | Update of TC 6.6.2.2A.2 for CA_2A-5A and CA_2A-12A | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170910 | 3314 | - | Update of TC 6.6.2.2A.2_1 for CA_2A-5A and CA_2A-12A | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170911 | 3315 | - | Correction of RB allocation in TC 6.2.5A.3 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170912 | 3316 | - | Correction of RB allocation in TC 6.6.2.1A.2 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170914 | 3317 | - | Corrected test requirements for TC 6.2.3A.2_1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170915 | 3318 | - | Correction to configuration table in TC 6.7A.2 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170919 | 3321 | - | Update of applicability for 4Rx CQI reporting under AWGN tests | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170949 | 3324 | - | Addition of CA_2A-66A and CA_5A-66A in TS 36.521-1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170957 | 3326 | - | Update to test case 7.6.1F In-band blocking for category NB1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170958 | 3327 | - | Update to test case 7.6.2F Out-of-band blocking for category NB1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-170996 | 3332 | - | New CA band combinations CA_1A-18A, 1A-26A, 1A-28A, 1A-42A and 18A-28A - Updates of 7.3A.6 REFSENS | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171006 | 3334 | - | Update test configuration and requirement for CA_8B in clause 6 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171010 | 3336 | - | Update RF general section for V2X information | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171040 | 3347 | - | Update of test tolerance of 4-Rx TDD Type A demod test cases | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171085 | 3353 | - | Addition of applicability and test rules for CA UE demodulation tests with 4DL and 5DL CCs | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171092 | 3360 | - | Addition of CA configuration CA_4A-5A-29A for Refsens test | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171094 | 3362 | - | Addition of CA configuration CA_20A-31A for Refsens test | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171095 | 3363 | - | Addition of CA configuration CA_7A-42A and CA_28A-40A for Refsens test | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171097 | 3365 | - | Addition of for CA_20A-42A and CA_20A-42A-42A Refsens test | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171098 | 3366 | - | Addition of for CA_40C Set 1 Refsens test | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171153 | 3368 | - | Update receiver test cases to remove DCI format 1C | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171161 | 3369 | - | Editorial correction in reference clause numbering for TC 6.3.4A.1.1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171162 | 3370 | - | Update test configuration table and requirement for CA_8B in clause 7 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171165 | 3371 | - | Addition of 7.4A.3_H and 7.4A.4_H to Annex F | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171167 | 3373 | - | Clean up on clause 7 256QAM test cases | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171178 | 3382 | - | Addition of Downlink Reference measurement channel for Cat-M1 TX characteristics | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171182 | 3383 | - | Clean up on Tx test cases for UE category M1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171183 | 3384 | - | Correction to general and additional Tx spurious for UE category M1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171184 | 3385 | - | Correction to HD-FDD PDSCH scheduling pattern for UE category M1 RMC | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171185 | 3386 | - | Correction to PUSCH-EVM with exclusion period for UE category M1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171186 | 3387 | - | Correction to Power control test cases for UE category M1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171187 | 3388 | - | Correction to ON/OFF time mask for UE category M1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171188 | 3389 | - | Clarification of test requirement for test cases 6.2.2A.1, 6.2.4A.1 and 6.2.4A.1_1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171190 | 3391 | - | Correction to additional spurious emissions test | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171191 | 3392 | - | Correction to test procedure for test case 7.9A | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171201 | 3396 | - | Editorial correction in test case 6.3.5A.2.2 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171202 | 3397 | - | Alignment frequency ranges in TC 6.6.3.1A.2 for CA_3A-26A and CA_5A-17A to TR 36.905 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171203 | 3398 | - | Correction in configuration IDs for NS_24 in test cases 6.2.4_2 and 6.6.3.3_1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171226 | 3401 | - | Update of Annex C to include 4 antenna ports in UE | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171227 | 3402 | - | Adding support for 4 Rx antenna port UEs in Receiver test cases | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171234 | 3406 | - | Update of Reference Sensitivity test case for UE category M1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171237 | 3409 | - | Correction to test case 7.3F.2, Reference sensitivity level with repetitions for category NB1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171238 | 3410 | - | Correction to test case header in 8.11.1.1.1, 8.11.1.1.2, 8.11.1.1.3 and 8.11.1.1.3_1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171247 | 3419 | - | Update of Frequency error test case for UE category M1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171315 | 3435 | - | Clean up in test case 6.6.2.1A.2 to remove additional spectrum emission requirements | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171349 | 3437 | - | Correction to NS_24 and NS_25 test configurations to align use of Ful | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171514 | 3439 | - | Update TT of the TC 8.2.3.1.1.3 TDD FDD CA PDSCH Single Antenna Performance for FDD PCell(4DL CA) | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171700 | 3248 | 1 | Clean up editor's note for UL 64QAM | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171701 | 3390 | 1 | Correction to UL CA Tx tests | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171707 | 3230 | 1 | Updates of 7.3A Refsens for CA_1C-3A | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171708 | 3297 | 1 | Introduction of CA_3A-11A to section 7 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171709 | 3298 | 1 | Introduction of CA_8A-28A to section 7 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171711 | 3300 | 1 | Introduction of 3DL/1UL of CA_1A-8A-28A to section 7 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171714 | 3301 | 1 | Introduction of 3DL/1UL of CA_3A-8A-28A to section 7 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171717 | 3302 | 1 | Introduction of 3DL/1UL of CA_3A-28A-41A to section 7 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171719 | 3438 | 1 | CA_29A-66A, CA_29A-66A-66A, CA_29A-66C, CA_46A-66A Reference Sensitivity requirements | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171725 | 3330 | 1 | Updates of 6.6.3.2 Spurious emission band UE co-existence for Band 5 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171733 | 3322 | 1 | LAA: Applicability and test selection update to Section 8 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171734 | 3270 | 1 | Annexure update for LAA RF | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171735 | 3328 | 1 | Update of test case 7.6.1A.3 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171736 | 3367 | 1 | Update test case 7.5A.3 to add LAA band combinations | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171748 | 3404 | 1 | Test tolerance for NAICS demod test cases with CRS interference | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171752 | 3329 | 1 | Updates to test tolerances and test uncertainty for 4Rx TCs | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171753 | 3271 | 1 | LAA Uncertainties and Test Tolerances for Test Case 7.8.1A.3 Intermodulation for CA, inter band DL CA | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171754 | 3320 | 1 | LAA: Test case and test tolerance addition for Dual Layer DM-RS demod | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171787 | 3265 | 1 | Update to MPDCCH demod test cases in CE Mode A | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171788 | 3289 | 1 | Correction of DCI format in MTCe2 TCs and PDSCH and MPDCCH configuration | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171790 | 3378 | 1 | Clean up on TRx TCs for UE category M1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171791 | 3379 | 1 | Correction to Carrier leakage and In-band emissions for Cat-M1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171792 | 3380 | 1 | Correction to Carrier leakage and In-band emissions | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171793 | 3412 | 1 | New eMTC test case 9.8.2.1, FDD and half-duplex FDD UE-selected subband CQI for UE category M1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171794 | 3414 | 1 | New eMTC test case 9.8.2.2, TDD UE-selected subband CQI for UE category M1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171795 | 3416 | 1 | New eMTC test case 9.8.1.2, TDD CQI reporting definition under AWGN conditions for UE category M1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171796 | 3408 | 1 | Correction to test case 9.8.1.1, FDD and Half duplex FDD CQI reporting definition under AWGN conditions for UE category M1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171813 | 3266 | 1 | Introduction of FDD and Half-Duplex FDD Demodulation of MPDCCH in CE Mode B Test Case | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171814 | 3267 | 1 | Introduction of TDD Demodulation of MPDCCH in CE Mode B Test Case | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171817 | 3235 | 1 | Addition of bandwidth combinations 10+5+10MHz and 20+10+5+10MHz in Chapter 8 FDD 3CA and 4CA test cases | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171818 | 3237 | 1 | Addition of bandwidth combinations 10+10+ 5MHz and 20+10+10+5MHz in TC 9.6.1.1_A.2 and 9.6.1.1_A.3 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171819 | 3240 | 1 | Addition of CA_2A-5A-12B test configuration and reference sensitivity in TC 7.3A.9. | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171821 | 3233 | 1 | Addition of CA_1A-3A-28A in 36.521-1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171823 | 3241 | 1 | Addition of CA_2A-4A-30A test configuration and reference sensitivity in TC 7.3A.5. | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171825 | 3243 | 1 | Addition of CA_2A-4A-12A-30A test configuration and reference sensitivity in TC 7.3A.9. | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171829 | 3242 | 1 | Addition of CA_5A-12B test configuration and reference sensitivity in TC 7.3A.5. | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171832 | 3260 | 1 | Addition of CA_2A-4A-5A-12A test configuration and reference sensitivity in TC 7.3A.9 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171834 | 3261 | 1 | Addition of CA_2C-12A-30A test configuration and reference sensitivity in TC 7.3A.9 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171837 | 3263 | 1 | Addition of CA_2C-30A test configuration and reference sensitivity in TC 7.3A.5 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171838 | 3255 | 1 | Addition of CA_2A-2A-4A-5A in subclause 7.3A.9 of 36.521-1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171839 | 3256 | 1 | Addition of CA_2A-2A-5A-12A in subclause 7.3A.9 of 36.521-1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171840 | 3257 | 1 | Addition of CA_2A-2A-12B in subclause 7.3A.9 of 36.521-1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171842 | 3308 | 1 | Update of TC 6.2.4A.2 for CA_2A-5A and CA_2A-12A | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171844 | 3331 | 1 | CA band combinations 1A-18A, 1A-26A, 1A-28A, 1A-42A and 18A-28A - Updates of 6.6.3.2A Spurious emission band UE co-existence | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171846 | 3309 | 1 | Update of TC 6.2.4A.2_1 for CA_2A-5A and CA_2A-12A | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171847 | 3311 | 1 | New test points in transmit modulation tests for CA_2A-5A and CA_2A-12A | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171848 | 3426 | 1 | Update of 4DL CA Test Cases | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171851 | 3341 | 1 | Addition of the new TC 8.2.3.1.1.4 TDD FDD CA PDSCH Single Antenna Performance for FDD PCell(5DL CA) | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171852 | 3351 | 1 | Addition of the new TC 8.7.5.1_H.4 TDD FDD CA Sustained data rate performance for FDD PCell (5DL CA) for 256QAM in DL | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171853 | 3375 | 1 | Clean up on clause 7 CA test cases | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171860 | 3433 | 1 | Correction to Rel 13 CA_5A-5A RF Test Cases Section 7 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171861 | 3358 | 1 | Addition of CA configuration CA_3A-40C, CA_3A-41C, CA_28A-40C for Refsens test | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171862 | 3359 | 1 | Addition of CA configuration CA_4A-28A for Refsens test | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171863 | 3361 | 1 | Addition of CA configuration CA_7A-42A-42A for Refsens test | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171864 | 3342 | 1 | Addition of the new TC 8.2.3.1.2.3 TDD FDD CA PDSCH Single Antenna Performance for TDD PCell(4DL CA) | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171865 | 3343 | 1 | Addition of the new TC 8.2.3.1.2.4 TDD FDD CA PDSCH Single Antenna Performance for TDD PCell(5DL CA) | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171866 | 3344 | 1 | Addition of the new TC 8.2.2.3.1_A.3 TDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (4DL CA) | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171867 | 3354 | 1 | Addition of for FDD PDSCH Single Antenna Port Performance for CA (4DL CA) (8.2.1.1.1_A.4) | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171868 | 3355 | 1 | Addition of for FDD PDSCH Single Antenna Port Performance for CA (5DL CA) (8.2.1.1.1_A.5) | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171869 | 3356 | 1 | Addition of for FDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (4DL CA) (8.2.1.3.1_A.3) | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171870 | 3357 | 1 | Addition of for FDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (5DL CA) (8.2.1.3.1_A.4) | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171871 | 3372 | 1 | Clarification of test points for sustained data rate test | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171872 | 3400 | 1 | Introduction of new Dual Connectivity test cases - Power Control Relative power tolerance | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171875 | 3282 | 1 | Addition of Band 47 for UE maximum output power for V2X Communication | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171876 | 3284 | 1 | Introduction of UE maximum output power with additional requirements for V2X Communication | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171877 | 3285 | 1 | Introduction of 6.2.5G Configured transmitted power for V2X Communication | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171878 | 3286 | 1 | Introduction of 6.3.4G.1 General ON/OFF time mask for V2X Communication | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171879 | 3337 | 1 | Add Operating bands, channel bandwidth and RMC for V2X | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171880 | 3427 | 1 | Introduction of Reference sensitivity level for V2X | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171881 | 3428 | 1 | Introduction of Maximum input level for V2X | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171882 | 3429 | 1 | Introduction of Adjacent Channel Selectivity (ACS) for V2X | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171883 | 3430 | 1 | Introduction of In-band blocking for V2X | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171884 | 3431 | 1 | Introduction of Out-of-band blocking for V2X | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171885 | 3432 | 1 | Introduction of Spurious response for V2X | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171887 | 3288 | 1 | Correction of payload in fixed reference channel for CDM-multiplexed DM RS with four CSI-RS antenna ports | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171888 | 3350 | 1 | Updates to legacy CSI test cases to include 4Rx UE | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171889 | 3376 | 1 | Correction to 3 or 4 layers 4Rx demodulation tests | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171890 | 3403 | 1 | Introduction of TDD 4Rx PDSCH demodulation test cases | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171896 | 3303 | 1 | Power Class 2: Update to A-MPR for NS_04 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171897 | 3268 | 1 | Updates of the SDR TC 8.7.5.2.2 and 8.7.5.2_H.2 for TDD FDD CA | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171898 | 3425 | 1 | Update of test configuration table of 4DL CA | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171916 | 3236 | 1 | TDD FDD CA Sustained data rate performance for TDD PCell (4DL CA) | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171917 | 3424 | 1 | Update of test configuration table of 3DL CA | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171918 | 3281 | 1 | Introduction to Performance test case 8.7.5.1.4 TDD FDD CA Sustained data rate performance for FDD PCell (5DL CA) | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171919 | 3345 | 1 | Introduction of LAA Closed-Loop spatial multiplexing performance test cases for 4Tx antenna ports | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171921 | 3348 | 1 | Test Tolerance updates to CQI reporting with enhanced receiver typeB test case | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171922 | 3246 | 1 | Update of NPDCCH single-antenna demodulation test | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171923 | 3247 | 1 | Update of NPDSCH demodulation test in standalone and Guard-band | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171924 | 3339 | 1 | Updates to In-band mode NPDSCH and NPDCCH Demodulation Performance tests for category NB1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171932 | 3381 | 1 | Correction to PDSCH demodulation test cases for UE category M1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171934 | 3299 | 1 | Introduction of CA_11A-28A to section 7 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171936 | 3283 | 1 | Introduction of UE maximum output power for modulation / channel bandwidth for V2X Communication | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171937 | 3231 | 1 | Update TS 36.521-1 with Addition of LTE Band 48 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171938 | 3399 | 1 | Addition configuration IDs to test case 6.2.3A.1_1 for CA configuration 10+20 MHz | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171940 | 3393 | 1 | Correction to TDD FDD 2CA Soft buffer test cases | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171941 | 3352 | 1 | Add minimum testing time in Annex G for SCE performance requirements | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171942 | 3349 | 1 | Updates to legacy demod test cases to include 4Rx UE | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171945 | 3440 | - | Band 41 HPUE Maximum Output Power | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171946 | 3441 | - | Adjacent Channel Leakage power Ratio for HPUE | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171949 | 3442 | 1 | Update of RMCs for NB-IoT RF tests | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171963 | 3275 | 2 | Update to test case 6.3.5F.2 Power Control Relative power tolerance for UE category NB1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171967 | 3377 | 2 | Correction to Aggregate power control tolerance for UE category M1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171968 | 3417 | 2 | Update of RMCs for UE category M1 in Annex A | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171969 | 3420 | 2 | Minimum requirements updates in section 8.11 test case for UE category M1 | 14.1.0 | 14.2.0 |
| 2017-03 | RAN#75 | R5-171997 | 3405 | 2 | Introduction of LAA PDCCH/PCFICH demodulation test case with FDD Pcell | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172116 | 3447 |  | NB-IoT bands 11, 25, 31, and 70 introduction to receiver requirements | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172142 | 3450 |  | Uncertainties and Test Tolerances for new LAA Test Cases - CQI reporting / PUSCH 3-1 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172196 | 3457 |  | Correction to typos in UL-MIMO test cases 6.3.2B | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172198 | 3458 |  | Clarification to measurement of SEM in test procedure of TC 6.6.2.1A.2 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172199 | 3459 |  | Introduction of switched case for inter-band CA transmit modulation test cases | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172200 | 3460 |  | Introduction of switched case for TC 6.5.2A.1.2_1 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172201 | 3461 |  | Update of test requirements in TC 7.3A.3 for missing CA configurations | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172203 | 3463 |  | Update of TC 7.3_1 to align with minimum conformance requirements | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172206 | 3464 |  | Update of TC 6.6.2.2A.2 for CA_2A-5A and CA_2A-12A | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172207 | 3465 |  | Update of TC 6.6.3.3A.2 for CA_2A-5A and CA_2A-12A | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172214 | 3467 |  | Remove content of SDR test cases for 4Rx | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172216 | 3468 |  | Update of requirements in TC 6.2.2A.2 for new Rel-14 CA configurations | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172219 | 3471 |  | Update of test requirements in TC 6.6.3.1A.2 for new Rel-14 CA configurations | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172221 | 3473 |  | Update of initial conditions in TC 6.6.3.3A.2 for new Rel-14 CA configurations | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172224 | 3476 |  | Update of test requirements in TC 7.3A.6 for new Rel-14 CA configurations | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172255 | 3477 |  | Update of the TC about TDD FDD CA Sustained data rate performance | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172260 | 3478 |  | Update of the TC 7.3A.5 REFSENS for 3DL CA (CA_1A-19A-42A, 1A-21A-42A, 19A-21A-42A) | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172307 | 3488 |  | Update to the Dual Connectivity performance test cases | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172345 | 3494 |  | Alignment of Fixed Reference Channel two antenna ports for UE category M1 to TS 36.101 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172347 | 3495 |  | Invalid csi-ReportMode-r10 in message exceptions of CSI reporting for 4Rx UE TC | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172350 | 3498 |  | Correction of UL RMC for 5+10+10 MHz test step in TC 8.7.1.1_A.2 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172351 | 3499 |  | Editorial correction of references to tables in clause UE category M1 TCs | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172352 | 3500 |  | Editorial correction of test case heading | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172353 | 3501 |  | Removal of duplicate Test step in TC 8.2.3.1.2.3 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172359 | 3504 |  | Update to chapter 8 & 9 PCC/SCC frequency mapping | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172360 | 3505 |  | Editorial update to Annex G for DL 256QAM | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172362 | 3506 |  | New eMTC test case 6.5.1EA_1, Frequency Error for UE category M1 (CE Mode B) | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172374 | 3516 |  | Align CA capability definitions for demodulation test cases in sub-clause 8.1.2.2 with TS 36.101 v14.3.0 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172375 | 3517 |  | Align CA delta RIB in sub-clause 7.3.3 with TS 36.101 v14.3.0 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172376 | 3518 |  | Align CA delta TIB in sub-clause 6.2.5.3 with TS 36.101 v14.3.0 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172377 | 3519 |  | Align CA minimum requirements for reference sensitivity in sub-clause 7.3A.0 with TS 36.101 v14.3.0 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172392 | 3521 |  | Updated to NB-IoT RF tests cases | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172397 | 3524 |  | Updated to Table of E-UTRA channel numbers for SDL Band 69 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172422 | 3526 |  | Resubmission to update Annex F for test case 7.5A.3 to add LAA band combinations | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172431 | 3528 |  | Correction of codebookSubsetRestriction in test case 9.9.1.3.2 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172433 | 3529 |  | Updates to FDD and half-duplex FDD demodulation of MPDCCH in CE Mode A and TDD demodulation of MPDCCH in CE Mode A | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172463 | 3540 |  | Addition of Annex F for TDD FDD CA PDSCH Open Loop Multiplexing 2x2 Performance Test cases (4 & 5DL CA) | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172487 | 3544 |  | Change of UE target power configuration for Carrier leakage and In-band emissions tests | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172527 | 3551 |  | Introduction of CA_8A-11A-28A to section 7 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172602 | 3578 |  | Addition of Annex G.3A.5 and G.3A.6 for 4DL CA test cases. | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172603 | 3579 |  | Removing of redundant editor's note for 4DL CA test cases | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172626 | 3585 |  | Correction to message contents for 4DL CA Rx tests | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172628 | 3587 |  | Correction to message contents of 4DL CA 8.2.1.4.2_A.3 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172634 | 3589 |  | Introducing uncertainty of receiver spurious emission for band 46 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172641 | 3594 |  | Correction to Annex A.2 and A.3 for Cat-M1 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172644 | 3597 |  | Correction to Annex A.4 for Cat-M1 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172650 | 3601 |  | Correction to spurious emission band UE co-existence test | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172651 | 3602 |  | Editorial correction to 6.2.5 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172652 | 3603 |  | Correction to message contents of 3DL CA Rx tests | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172656 | 3607 |  | Correction to message contents for 2DL CA Rx tests | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172658 | 3609 |  | Correction to PMI test for TM9 with 8 TX and with 4TX enhanced codebook | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172710 | 3610 |  | Test tolerance and statistical test time addition to 4Rx demodulation test cases | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172715 | 3611 |  | Addition of NS_27 for Band 48 in A-MPR test cases | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172718 | 3614 |  | Introduction of test case LAA PCFICH/PDCCH Transmit Diversity 2x2 with TDD as Pcell | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172720 | 3616 |  | Applicability correction for eDL-MIMO test cases in part 1 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172724 | 3617 |  | Correction to test case references in tables for applicability and test rules in clause 8.1.2.3 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172814 | 3625 |  | Update of minimum conformance requirement in 7.4A.0 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172824 | 3628 |  | Introduction of 8.2.3.2.1.3 TDD FDD CA PDSCH Open Loop Multi Layer Spatial Multiplexing 2x2 for FDD Pcell (4DL CA) | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-172825 | 3629 |  | Introduction of 8.2.3.2.1.4 TDD FDD CA PDSCH Open Loop Multi Layer Spatial Multiplexing 2x2 for FDD Pcell (5DL CA) | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173052 | 3621 | 1 | Updates to legacy CA CQI reporting test cases to include 4Rx UE | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173053 | 3622 | 1 | Updates to legacy single carrier CQI reporting test cases to include 4Rx UE | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173054 | 3623 | 1 | Updates to legacy PMI and RI test cases to include 4Rx UE | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173063 | 3595 | 1 | Correction to CSI tests for Cat-M1 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173064 | 3596 | 1 | Correction to MPDCCH aggregation level for Cat-M1 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173065 | 3598 | 2 | Correction to Cat-M1 demodulation test cases | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173070 | 3550 | 2 | Introduction of CA_1A-11A-28A to section 7 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173071 | 3557 | 2 | Update of 2DL/1UL of CA_3A-3A and CA_7A-7A to section 7.3A.3 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173072 | 3558 | 2 | Introduction of 3DL/1UL of CA_3A-3A-7A and CA_3A-7A-7A to section 7.3A.5 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173073 | 3559 | 2 | Introduction of 4DL/1UL of CA_3A-3A-7A-7A to section 7.3A.9 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173200 | 3523 | 1 | Updated Test Applicability for TC 6.2.3E | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173201 | 3456 | 1 | Correction to typos in Rx test cases for CA | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173202 | 3522 | 1 | Updated table references related to reference sensitivity in test description and test requirements | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173203 | 3604 | 1 | Correction to UL configuration for REFSENS test for CA_41C and CA_7C | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173204 | 3605 | 1 | Clean up on test configuration tables for intra-contiguous 2CA Rx tests | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173205 | 3493 | 1 | Update of NAICS demodulation TCs | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173206 | 3608 | 1 | Correction to Annex B.2.2 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173209 | 3443 | 1 | Update of test points and test requirements in TC 7.3A.5 for CA_4A-12A-30A | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173210 | 3451 | 1 | New CA band combinations CA_1A-41A-42C and 1A-41C-42A - Updates of 7.3A.9 REFSENS | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173212 | 3452 | 1 | New CA band combinations CA_3C-8A - Updates of 7.3A.5 REFSENS | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173215 | 3472 | 1 | Update of TC 6.6.3.2A.2 for Rel-14 CA configuration CA_2A-7A | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173216 | 3520 | 1 | Align operating bands and Channel bandwidths in sub-clause 5.2, 5.2A, 5.4.2.1 and 5.4.2A with TS36.101 v14.3.0 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173217 | 3527 | 1 | Addition of CA_13A-66A in TS 36.521-1 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173220 | 3552 | 1 | Corrections on 3A-28A-41A in Test Configuration Table of section 7.3A.5.4.1 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173221 | 3554 | 1 | Introduction of 2UL/2DL CA_41A-42A to section 7 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173223 | 3600 | 1 | Addition of CA configuration CA_66A-66C, CA_xA-66A-66A and CA_66A-66A-xA for Refsens test | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173228 | 3624 | 1 | Test Tolerances and general updates to NB-IoT performance test cases | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173276 | 3448 | 1 | Addition of new LAA Test Case - CQI reporting / PUSCH 3-1 / FDD Pcell | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173277 | 3449 | 1 | Addition of new LAA Test Case - CQI reporting / PUSCH 3-1 / TDD Pcell | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173278 | 3525 | 1 | Introduction of LAA Dual-Layer Spatial Multiplexing with DM-RS with TDD as Pcell Test Case | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173279 | 3542 | 1 | Update test case 7.6.2A.3 to add LAA band combinations | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173280 | 3543 | 1 | Update test case 7.7A.3 to add LAA band combinations | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173281 | 3626 | 1 | LAA: Addition of CQI Reporting with FDD as PCELL | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173288 | 3453 | 1 | Introduction to New Rx test case 7.6.3A.8 Narrow band blocking for 5DL CA | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173289 | 3454 | 1 | Introduction to New Rx test case 7.7A.8 Spurious response for 5DL CA | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173290 | 3455 | 1 | Introduction to New Rx test case 7.8.1A.8 Wideband intermodulation for 5DL CA | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173291 | 3466 | 1 | Update of test requirements in TC 7.3A.6 for new Rel-13 CA configurations | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173295 | 3535 | 1 | Introduction of 8.2.3.2.2.3 TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for TDD PCell (4DL CA) | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173296 | 3536 | 1 | Introduction of 8.2.3.2.2.4 TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for TDD PCell (5DL CA) | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173298 | 3560 | 1 | Correction of 3DL CA & 4DL CA RX TC | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173299 | 3561 | 1 | Introduction of New TC7.3A.10 Reference sensitivity level for 5DL CA | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173301 | 3562 | 1 | Addition of CA_8A-41C in test configuration in TC 7.3A.5 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173302 | 3539 | 1 | Addition of G.3A for TDD FDD CA PDSCH Single Antenna and Open Loop Spatial Multiplexing 2x2 Performance Test cases (4 & 5DL CA) | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173303 | 3555 | 1 | Update of 2DL/1UL of CA_3A-42A, CA_28A-42A, CA_41A-42A and CA_42A-46A to section 7.3A.3 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173305 | 3556 | 1 | Introduction of 3DL/1UL of CA_2A-4A-7A and CA_3A-41A-42A to section 7.3A.5 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173306 | 3618 | 1 | Addition of test case TDD FDD CQI Reporting under AWGN conditions – PUCCH 1-0 for FDD PCell (5DL CA) (9.6.1.3.4) | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173307 | 3619 | 1 | Addition of test case TDD FDD CQI Reporting under AWGN conditions – PUCCH 1-0 for TDD PCell (5DL CA) (9.6.1.4.4) | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173316 | 3510 | 1 | Updates to eMTC test case 8.11.1.1.3, FDD and half-duplex FDD PDSCH Transmit Diversity 2x1 for UE category M1 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173317 | 3514 | 1 | Updates to eMTC test case 8.11.1.2.3, TDD PDSCH Transmit Diversity for UE category M1 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173318 | 3515 | 1 | Updates to eMTC test case 9.8.1.1, FDD and Half duplex FDD CQI reporting definition under AWGN conditions for UE category M1 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173329 | 3496 | 1 | Sending of MaxLayersMimo-r10 IE for 4Rx TCs with more than 2 Layers | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173330 | 3530 | 1 | Updates to legacy FDD PDSCH demod test cases to include 4Rx UE | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173331 | 3531 | 1 | Updates to legacy TDD PDSCH demod test cases to include 4Rx UE | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173332 | 3532 | 1 | Updates to legacy TDD FDD CA PDSCH demod test cases to include 4Rx UE | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173333 | 3533 | 1 | Updates to legacy sustained data rate test cases to include 4Rx UE | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173339 | 3612 | 1 | Addition of NS_27 for Band 48 in A-SEM test cases | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173340 | 3613 | 1 | Addition of NS_27 for Band 48 in A-Spurious test cases | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173341 | 3444 | 1 | Correction to Band 70 test frequencies to clarify the use of two duplex spacings | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173342 | 3508 | 1 | Updates to eMTC test case 8.11.1.1.1, FDD and half-duplex FDD Closed-loop spatial multiplexing performance for UE category M1 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173343 | 3509 | 1 | Updates to eMTC test case 8.11.1.1.2, FDD and half-duplex FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category M1 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173344 | 3512 | 1 | Updates to eMTC test case 8.11.1.2.1, TDD Closed-loop spatial multiplexing performance for UE category M1 (Cell-Specific Reference Symbols) | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173345 | 3513 | 1 | Updates to eMTC test case 8.11.1.2.2, TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category M1 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173346 | 3497 | 1 | eHST RF: Practical and consistent model definition for HST-SFN scenario | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173347 | 3549 | 1 | Introduction of reference channels for enhanced high speed train test | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173348 | 3481 | 1 | Update of the TC 6.6.3F.2 Spurious emission band UE co-existence for category NB1 for Release 14 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173349 | 3581 | 1 | NB-IoT band 70 introduction to transmitter requirements | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173351 | 3446 | 1 | CA_46A-46A-66A, CA_46C-66A Receiver requirements | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173354 | 3482 | 1 | Update to operating bands for V2X Communication | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173355 | 3483 | 1 | Introduction of 6.2.2G UE Maximum Output Power for V2X Communication | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173356 | 3484 | 1 | Update to 6.2.3G.1 Maximum Power Reduction (MPR) for V2X Communication / Power class 3 / Contiguous allocation of PSCCH and PSSCH | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173357 | 3485 | 1 | Update to 6.2.4G.1 Additional Maximum Power Reduction (A-MPR) for V2X Communication / Non-concurrent with E-UTRA uplink transmission | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173358 | 3486 | 1 | Update to TC 6.2.5G.1 Configured UE transmitted Output Power for V2X Communication / Non-concurrent with E-UTRA uplink transmission | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173359 | 3489 | 1 | Introduction of 6.2.2G.2 UE Maximum Output Power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173360 | 3490 | 1 | Introduction of 6.2.5G.2 Configured UE transmitted Output Power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173361 | 3492 | 1 | Introduction of 6.2.3G.1.3 Maximum Power Reduction (MPR) for V2X Communication / Power class 3 / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173370 | 3566 | 1 | Update of 7.3G.1 Reference sensitivity level for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173372 | 3568 | 1 | Introduction of 7.3G.3 Reference sensitivity level for V2X Communication / Intra-band contiguous MCC operation | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173373 | 3569 | 1 | Update of 7.4G.1 Maximum input level for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173374 | 3570 | 1 | Introduction of 7.4G.2 Maximum input level for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173375 | 3571 | 1 | Introduction of 7.4G.3 Maximum input level for V2X Communication / Intra-band contiguous MCC operation | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173376 | 3572 | 1 | Update of 7.5G.1 Adjacent channel selectivity (ACS) for V2X Communication / Non-concurrent | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173377 | 3573 | 1 | Introduction of 7.5G.2 Adjacent channel selectivity (ACS) for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173378 | 3574 | 1 | Introduction of 7.5G.3 Adjacent channel selectivity (ACS) for V2X Communication / Intra-band contiguous MCC operation | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173379 | 3575 | 1 | Update of 7.6.1G.1 In-band blocking for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173380 | 3576 | 1 | Introduction of 7.6.1G.2 In-band blocking for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173381 | 3577 | 1 | Introduction of 7.6.1G.3 In-band blocking for V2X Communication / Intra-band contiguous MCC operation | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173382 | 3582 | 1 | Introduction of TC 7.8.1G Wide band Intermodulation for V2X | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173383 | 3583 | 1 | Introduction of TC7.9G Spurious emissions for V2X | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173384 | 3487 | 1 | Correction to the A-SE requirement for Band 41 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173385 | 3469 | 1 | Update of TC 6.2.4A.2 for new Rel-14 CA configurations | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173386 | 3470 | 1 | Update of TC 6.2.4A.2_1 for new Rel-14 CA configurations | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173387 | 3474 | 1 | Update of TC 6.6.2.2A.2 for new Rel-14 CA configurations | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173388 | 3475 | 1 | Update of TC 6.6.2.2A.2_1 for new Rel-14 CA configurations | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173390 | 3553 | 1 | Introduction of 2UL/2DL CA_41A-42A to section 6 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173395 | 3564 | 1 | Addition of CA bands in test configuration in TC 7.3A.5 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173396 | 3565 | 1 | Addition of CA bands in test configuration in TC 7.3A.9 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173397 | 3580 | 1 | Introduction of 3DL/1UL of CA_3A-20A-32A to section 7 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173399 | 3563 | 1 | Addition of CA_12A-66A in test configuration in 7.3A.3 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173400 | 3545 | 1 | Change of UE target power configuration for EVM test | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173401 | 3479 | 1 | Update of the TC 7.3A.9 REFSENS for 4DL CA (CA_1A-3A-19A-42A, 1A-19A-21A-42A, 1A-19A-42C, 1A-21A-42C, 19A-21A-42C) | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173402 | 3534 | 1 | Correction to Test configuration Tables in subclause 7.3A for 3DL CA and 4DL CA | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173403 | 3584 | 1 | Correction to 4DL CA out of band blocking interferer | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173404 | 3620 | 1 | Correction of CA configuration CA_28A-40C Refsens test | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173409 | 3591 | 1 | Clean up on Cat-M1 Rx test cases | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173410 | 3592 | 1 | Clean up on Cat-M1 Tx test cases | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173411 | 3593 | 1 | Correction to spurious emission band UE co-existence for UE category M1 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173417 | 3462 | 1 | Update of legacy Reference Sensitivity CA tests for 4Rx | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173421 | 3507 | 1 | Updates to Annex A and Annex G for eMTC test cases | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173422 | 3511 | 1 | Updates to eMTC test case 8.11.1.1.3_1, FDD PDS and half-duplex FDD CH Transmit Diversity 2x1 for UE category M1 (CEmodeB) | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173423 | 3546 | 1 | Clean up NPDCCH single-antenna performance test | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173424 | 3547 | 1 | Clean up NPDSCH demodulation test in standalone and Guard-band mode | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173427 | 3606 | 1 | Correction to 3DL CA out of band blocking interferer | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173428 | 3590 | 1 | Correction to transmit signal quality tests for category M1 | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173431 | 3502 | 1 | Updates to A-MPR, A-SEM and A-Spur TCs | 14.2.0 | 14.3.0 |
| 2017-06 | RAN#76 | R5-173436 | 3567 | 2 | Introduction of 7.3G.2 Reference sensitivity level for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173529 | 3630 | - | Correction of Test Settings for CA_4A-12A-30A in TC 7.3A.5 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-173617 | 3637 | - | Addition of CA_2A-5A-66A in TS 36.521-1 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-173619 | 3639 | - | Addition of CA_2A-13A-66A in TS 36.521-1 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-173694 | 3645 | - | Addition of CA_XA-YC-ZA to 7.4A.7 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-173695 | 3646 | - | Addition of CA_XA-YC-ZA to 7.5A.7 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-173696 | 3647 | - | Addition of CA_XA-YC-ZA to 7.6.1A.7 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-173697 | 3648 | - | Addition of CA_XA-YC-ZA to 7.6.2A.7 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-173698 | 3649 | - | Addition of CA_XA-YC-ZA to 7.6.3A.7 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-173699 | 3650 | - | Addition of CA_XA-YC-ZA to 7.8.1A.7 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-173784 | 3652 | - | Editorial correction to 4Rx Applicability for CSI | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-173785 | 3653 | - | Addition of FS3 OCNG pattern to Rx tests | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-173787 | 3655 | - | Clarification on UL RMC for category NB1 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-173797 | 3658 | - | Addition of measurement period for 9.8.2.1 in Annex G.5.4 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-173798 | 3659 | - | Removing editor’s note regarding UE’s declaration on carrier center frequency for Cat M1 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-173912 | 3667 | - | Editorial: Addition of band 40 for eMTC | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-173942 | 3674 | - | Editorial: Addition of V2V reference resource pooll configurations | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-173949 | 3676 | - | Addition of new Rel-13 CA RF test case 7.4A.8 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-173962 | 3678 | - | Update to test requirements in TC 6.7 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-173963 | 3679 | - | Updates to test requirements for CA tests with interferers | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-173984 | 3683 | - | Correction of test procedure and test requirement in TC 6.6.3.2A.2 (editorial change) | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-173993 | 3686 | - | eMTC correction of missing references for UL RMCs | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-173994 | 3687 | - | Correction of test requirement in TC 7.3_1 (editorial change) | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-173999 | 3690 | - | New eMTC test case, 6.3.5EA.3_1 Aggregate power control tolerance for UE category M1 (CE Mode B) | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174006 | 3694 | - | Addition of new 4Rx SDR test cases - Annex F | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174007 | 3695 | - | Update Annex A with RMCs for 4Rx and 5MHz (editorial change) | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174024 | 3700 | - | Addition of new CA Configuration CA_3A-38A to TS 36.521-1 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174026 | 3701 | - | Editorial Change of TC 8.10.1.1.3 – Alignment to TS 36.101 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174027 | 3702 | - | Correction of RB allocation in TC 6.5.2.3EA | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174029 | 3704 | - | Editorial Change of RBstart expression in category M1 TC | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174031 | 3706 | - | Editorial Change of heading format for TC 6.3.3F | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174090 | 3713 | - | Editorial change of CA configuration for CA_41C-42A and CA_41C-42C | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174092 | 3714 | - | New CA band combination CA_41A-42C(2UL) and CA_41C_42A(2UL) - Updates of 7.3A.5 REFSENS | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174096 | 3716 | - | Correction of test frequency in 6.2.4EA for NS_17 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174097 | 3717 | - | Correction of test frequency in 6.6.3EA.3 for NS_17 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174135 | 3724 | - | Editorial Change to align LAA capability definitions for demodulation test cases in sub-clause 8.1.2 with TS 36.101 v14.4.0 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174136 | 3725 | - | Added CA band combination to test requirement | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174152 | 3729 | - | Addition of 1.4 and 3 MHz to Ch 5 in 36.521-1 for Band 65 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174153 | 3730 | - | Addition of 1.4 and 3 MHz to Ch 7 in 36.521-1 for Band 65 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174155 | 3731 | - | Receiver requirements for CA_29A-70A, CA_29A-46A-66A, CA_46A-66A-66A, CA_46A-66C, CA_46A-70A | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174178 | 3732 | - | Editorial Change of tables for channel bandwidths per operating band for CA | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174244 | 3738 | - | Introduction of the new TC7.5A.8 ACS for 5DL CA | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174245 | 3739 | - | Introduction of the new TC7.6.1A.8 In-band blocking for 5DL CA | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174246 | 3740 | - | Introduction of the new TC7.6.2A.8 Out-of-band blocking for 5DL CA | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174256 | 3743 | - | FDD Enhanced Downlink Control Channel Performance Type A for PCFICH/PDCCH, 2 Tx Antenna Port under Asynchronous Network | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174257 | 3744 | - | FDD Enhanced Downlink Control Channel Performance Type A for PCFICH/PDCCH, 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174258 | 3745 | - | Test procedure correction for CQI-RI test cases | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174261 | 3748 | - | TDD Enhanced Downlink Control Channel Performance Type A for PCFICH/PDCCH, 2 Tx Antenna Port with Colliding CRS Dominant Interferer | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174262 | 3749 | - | TDD Enhanced Downlink Control Channel Performance Type A for PCFICH/PDCCH, 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174264 | 3751 | - | Addition of test points in test configuration table for RX sensitivity TCs | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174270 | 3755 | - | TDD Enhanced Downlink Control Channel Performance Type B for PCFICH/PDCCH, 2 Tx Antenna Port with Colliding CRS Dominant Interferer | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174271 | 3756 | - | TDD Enhanced Downlink Control Channel Performance Type B for PCFICH/PDCCH, 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174342 | 3769 | - | Introduction of new Dual Connectivity test cases - General ON/OFF time mask for Dual Connectivity | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174346 | 3772 | - | Updates to FDD and TDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174356 | 3774 | - | Editorial correction to Cat M1 MPDCCH test case | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174357 | 3775 | - | Editorial change to include note 2 reference for Band 22 tolerance in 6.2.2 UE Maximum Output Power | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174429 | 3785 | - | Editorial change to modify wrong references in category NB1 test cases | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174440 | 3789 | - | Addtion of test tolerance statement for TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 (4&5 DL CA) in Annex F | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-174680 | 3697 | 1 | Editorial Change - Correction of band sets for CA_7A-46C and CA_7A-46D | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175001 | 3681 | 1 | Addition of test requirement in TC 7.3A.5 for CA_2A-7A-12A | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175002 | 3720 | 1 | Update to Rx test case 7.6.3A.8 Narrow band blocking for 5DL CA | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175003 | 3721 | 1 | Update to Rx test case 7.7A.8 Spurious response for 5DL CA | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175004 | 3722 | 1 | Update to Rx test case 7.8.1A.8 Wideband intermodulation for 5DL CA | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175005 | 3726 | 1 | Editorial Change to align CA capability definitions for demodulation test cases in sub-clause 8.1.2.3 with TS 36.101 v14.4.0 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175006 | 3741 | 1 | Addition of the CA_1A-3A, 1A-19A, 1A-21A, 3A-19A, 19A-21A for 2UL 64QAM TCs | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175007 | 3765 | 1 | Updated RF general section 4.1 to align with core specification | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175008 | 3777 | 1 | Add New TC of 6.2.2A.4.1 of UE Maximum Output Power for intra-band contiguous 3 UL CA | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175009 | 3778 | 1 | Add New TC of 6.2.2A.4.2 for UE Maximum Output Power for inter-band 3 UL CA | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175010 | 3779 | 1 | Add New TC of 6.5.1A.4.1 for Frequency error for intra-band contiguous 3 UL CA | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175011 | 3780 | 1 | Add New TC of 6.5.1A.4.2 of Frequency error for inter- band 3 UL CA | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175012 | 3790 | 1 | Addtion of minimum test time statement for TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 (4&5 DL CA) in Annex G | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175025 | 3715 | 1 | New CA band combination CA_1A-41C-42C - Updates of 7.3A.10 REFSENS | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175026 | 3698 | 1 | Addition of a few Band 46 CA Configurations to TS 36.521-1 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175027 | 3712 | 1 | Addition of Rel-14 CA minimun and test requirements for Spurious emission band UE co-existence for CA | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175052 | 3666 | 1 | Editorial Change to TC7.3 minimum conformance requirements of 4Rx to same text in core spec | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175055 | 3767 | 1 | Introduction of A-SEM test case with PUSCH frequency hopping | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175056 | 3781 | 1 | Correction to test applicability of test cases 8.2.1.3.1 and 8.2.1.3.1_1 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175057 | 3685 | 1 | Correction of initial conditions in TC 6.6.2.2A.2_1 (editorial change) | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175058 | 3782 | 1 | Editorial Change - Reordering of Tables according to NS value | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175059 | 3708 | 1 | Editorial Change – Cleanup of Test Configuration Tables of TC 7.3A.5 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175060 | 3661 | 1 | Correction to TDD PMI Reporting Multiple PMI for eDL-MIMO test case 9.4.2.3.2_D | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175065 | 3657 | 1 | Correction to UE-selected subband CQI test for Cat M1 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175066 | 3696 | 1 | Correction of minimum test time for ETU1 in annex G | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175067 | 3705 | 1 | Editorial Change – Correction of minimum test time for TC 8.11.1.1.3 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175068 | 3773 | 1 | Completion of eMTC A-MPR test cases for NS_07 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175075 | 3692 | 1 | Addition of new 4Rx SDR test case - FDD 2DL | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175076 | 3693 | 1 | Addition of new 4Rx SDR test case - FDD 3DL | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175077 | 3746 | 1 | FDD Enhanced Downlink Control Channel Performance Type B for PCFICH/PDCCH, 2 Tx Antenna Port with Colliding CRS Dominant Interferer | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175078 | 3747 | 1 | FDD Enhanced Downlink Control Channel Performance Type B for PCFICH/PDCCH, 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175079 | 3768 | 1 | Introduction of new Dual Connectivity test cases - Power Control Relative power tolerance | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175085 | 3631 | 1 | Introduction of In-band emissions for non-allocated RB for V2X Communication / Non-concurrent with E-UTRA uplink transmissions in 6.5.2.3G.1 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175086 | 3633 | 1 | Update of TC7.6.2G Out-of-band blocking for V2X Communication | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175087 | 3634 | 1 | Update of TC 7.8.1G.1 Wide band Intermodulation for V2X | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175088 | 3635 | 1 | Update of TC7.9G Spurious emissions for V2X | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175089 | 3642 | 1 | Introduction of Spurious emission for V2X Communication / Non-concurrent with E-UTRA uplink transmissions in 6.6.3G.1 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175090 | 3675 | 1 | Addition of V2V new test case 6.5.1G.1 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175091 | 3718 | 1 | Adding new TC 6.6.1G.1 Occupied bandwidth for V2X Communication/ Non-concurrent with E-UTRA uplink transmissions | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175092 | 3723 | 1 | GNSS Reference signal for V2V | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175093 | 3733 | 1 | Introduction of Spurious emission band UE co-existence for V2X Communication / Non-concurrent with E-UTRA uplink transmissions in 6.6.3G.1_1 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175094 | 3764 | 1 | Introduction of Additional Spectrum Emission Mask for V2X Communication / Non-concurrent with E-UTRA uplink transmissions in 6.6.2.2G.1 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175095 | 3784 | 1 | Editorial: Addition of V2V RMC for performance requirements | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175096 | 3787 | 1 | New test cases: Demodulation performance Requirements for V2X Sidelink communications | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175102 | 3643 | 1 | Introduction of Spurious emission for V2X Communication / Intra-band contiguous MCC operation in 6.6.3G.3 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175103 | 3752 | 1 | Editorial change for V2X reference sensitivity TC | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175104 | 3753 | 1 | Update & editorial change for V2V RX Test Cases | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175105 | 3757 | 1 | Introduction ofSpurious emission band UE co-existence for V2X Communication / Intra-band contiguous MCC operation in 6.6.3G.3_1 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175106 | 3766 | 1 | Update RF general section 3 for V2X information | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175107 | 3791 | 1 | Introduction of Spectrum Emission Mask for V2X Communication / Intra-band contiguous MCC operation in 6.6.2.1G.3 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175111 | 3786 | 1 | Editorial change to align 7.8.1F minimun and test requirements with core spec | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175113 | 3750 | 1 | Addition of test requirement & test points in test configuration table for 3A-69A | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175114 | 3776 | 1 | Addition of Band 69 in Receiver test cases | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175115 | 3788 | 1 | Addition of Cat1bis demod test cases | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175122 | 3669 | 1 | New TC: FDD PDSCH performance in HST-SFN scenario | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175123 | 3670 | 1 | New TC: TDD PDSCH performance in HST-SFN scenario | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175125 | 3770 | 1 | Addition of Rel14 bands for NB-IoT in Clause 5 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175126 | 3771 | 1 | Addition of Rel14 bands in NB-IoT Transmitter test cases | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175127 | 3699 | 1 | Editorial Change – Cleanup of Test Configuration Tables of TC 7.3A.9 and TC 7.3A.10 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175128 | 3727 | 1 | Updated to test requirement to clause 8 test cases | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175129 | 3737 | 1 | Introduction of TDD CQI Reporting under AWGN conditions – PUCCH 1-0 for CA (4DL CA) | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175130 | 3742 | 1 | Update of the test requirement about TC 7.3A.10 REFSENS for 5DL CA | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175137 | 3662 | 1 | Introduction ofTDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for FDD PCell (4DL CA) in 8.2.3.3.1.3 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175138 | 3663 | 1 | Introduction of TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for FDD PCell (5DL CA) in 8.2.3.3.1.4 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175139 | 3664 | 1 | Introduction of TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for TDD PCell (4DL CA) in 8.2.3.3.2.3 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175140 | 3665 | 1 | Introduction of TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for TDD PCell (5DL CA) in 8.2.3.3.2.4 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175145 | 3673 | 1 | Addition of LAA test case 9.2.6.2 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175146 | 3758 | 1 | NBIoT correction of bandwidth | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175160 | 3689 | 1 | Correction to eMTC test case (CEModeB) to support repetition factor >1 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175161 | 3710 | 1 | Updates for FDD and HD-FDD and TDD Demodulation of MPDCCH in CE Mode B Test Cases | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175168 | 3644 | 1 | New CA band combinations CA_1A-3C-8A - Updates of 7.3A.9 REFSENS | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175169 | 3680 | 1 | Introduction of CA_1A-3A-11A to section 7 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175170 | 3682 | 1 | Introduction of CA_3A-8A-11A to section 7 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175171 | 3684 | 1 | Introduction of CA_3A-11A-28A to section 7 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175176 | 3783 | 1 | Update test case 7.6.2A.3 for LAA band combinations | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175177 | 3638 | 1 | Introduction of EVM equalizer spectrum flatness for V2X Communication / Non-concurrent with E-UTRA uplink transmissions in 6.5.2.4G.1 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175178 | 3640 | 1 | Introduction of Spectrum Emission Mask for V2X Communication / Non-concurrent with E-UTRA uplink transmissions in 6.6.2.1G.1 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175179 | 3636 | 1 | Introduction of In-band emissions for non-allocated RB for V2X Communication / Intra-band contiguous MCC operation in 6.5.2.3G.3 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175180 | 3719 | 1 | Adding new TC 6.6.1G.2 Occupied bandwidth for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175181 | 3754 | 1 | Update & editorial change for V2X RX Test Cases | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175185 | 3761 | 1 | Update to Configured UE transmitted Output Power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175190 | 3691 | 1 | Addition of new 4Rx SDR test case - FDD single carrier | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175191 | 3711 | 1 | Introduction of TDD-FDD CA PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model-Enhanced Performance Requirement Type A for TDD Pcell (2DL CA) | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175192 | 3734 | 1 | Introduction of TDD-FDD CA PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model-Enhanced Performance Requirement Type A for FDD Pcell (2DL CA) | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175197 | 3632 | 1 | Editorial correction to In-band emissions for non allocated RB in 6.5.2.3.4.2 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175199 | 3672 | 1 | Addition of eIMTA performance test case 8.2.2.2.5 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175203 | 3654 | 1 | Correction to NB-IoT test cases 6.5.1F and 6.6.2.1F | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175206 | 3735 | 2 | Introduction of TDD-FDD CA PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 Interference Model-Enhanced Performance Requirement Type A for TDD Pcell (2DL CA) | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175207 | 3736 | 2 | Introduction of TDD-FDD CA PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 Interference Model-Enhanced Performance Requirement Type A for FDD Pcell (2DL CA) | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175208 | 3688 | 1 | Editorial change, correction of RMC for Cat-M1 TDD PDSCH demodulation requirements | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175210 | 3707 | 1 | Update of TC 6.3.4F ON/OFF time mask for category NB1 | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175217 | 3656 | 1 | Correction to Test Parameters for Cat M1 CQI tests | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175220 | 3759 | 2 | Update to 6.2.2G.2 UE Maximum Output Power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission | 14.3.0 | 14.4.0 |
| 2017-09 | RAN#77 | R5-175221 | 3760 | 2 | Update to Maximum Power Reduction (MPR) for V2X Communication / Power class 3 / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176118 | 3793 | - | Correction of eMTC minimum test time | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176196 | 3806 | - | Update of Annexes F.1.4 and F.3.4 | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176229 | 3809 | - | Introduction of New TC 6.5.2A.1.4 Error Vector Magnitude (EVM) for CA (3UL CA) | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176230 | 3810 | - | Introduction of New TC 6.5.2A.2.4 Carrier leakage for CA (3UL CA) | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176231 | 3811 | - | Introduction of New TC 6.5.2A.3.4 In-band emissions for non allocated RB for CA (3UL CA) | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176245 | 3818 | - | Correction of eMTC test case 7.3EA | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176246 | 3819 | - | Aligning CA delta TIB in sub-clause 6.2.5.3 with TS 36.101 v14.5.0 | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176247 | 3820 | - | Aligning CA delta RIB in sub-clause 7.3.3 with TS 36.101 v14.5.0 | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176252 | 3822 | - | Adding missing ULCA bandwidth class B MPR | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176299 | 3827 | - | Updated Annex I.2.2 Tests for CA | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176300 | 3828 | - | Updated Annex G for new 4DL CA test cases | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176301 | 3829 | - | Added TC7.3A.9 test requirement for CA_2A-2A-5A-30A | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176308 | 3832 | - | Updated to Annex A for RMC and reference resource pool configuration | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176336 | 3846 | - | Addition of new 4Rx SDR test cases - Annex F | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176386 | 3850 | - | New CA band combinations CA_1A-3A-8A-40A - Updates of 7.3A.9 REFSENS | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176388 | 3851 | - | Updates of the SDR TC 8.7.5.1_H.3 for TDD FDD CA | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176418 | 3860 | - | Introduction of Spurious response for V2X in 7.7G.2 | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176423 | 3862 | - | Update of 4DL CA Test Cases | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176425 | 3863 | - | Correction to applicability statement of 4Rx CQI test cases | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176430 | 3864 | - | Update the applicability of test points for CA configurations 3A-3A and 7A-7A | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176431 | 3865 | - | Update REFSENS of 3DL CA for multiple CA configurations in 7.3A.5 | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176432 | 3866 | - | Addition of REFSENS of 2A-4A-7A-7A in 7.3A.9 | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176453 | 3869 | - | Update of UE co-existence requirements for Band 39 in 6.6.3.2 | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176477 | 3870 | - | Missed Addition of Cat1bis demod test case | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176493 | 3875 | - | Correction to test procedure for 6.3.5F.2 | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176496 | 3878 | - | Correction to 6.5.2.1F.1 and 6.5.2.3F | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176503 | 3881 | - | Correction to requirement of 9.4.1.3.2_D | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176504 | 3882 | - | Editorial correction to 8.2.1.4.2_A.3 | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176505 | 3883 | - | Clean up on LAA demodulation test cases | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176507 | 3885 | - | Correction to the test applicability for 2Rx SDR tests (36.521-1) | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176541 | 3889 | - | Introduction of TDD FDD CA Sustained data rate performance for TDD PCell (5DL CA) in 8.7.5.2.4 | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176553 | 3892 | - | Correction of test points for intra band non-contiguous 2DL CA Max input level | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176555 | 3894 | - | Editorial correction of title for 4Rx chapter 9 TCs in 36.521-1 | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176559 | 3896 | - | Alignment of TC 9.9.2.2.1 & 9.9.2.2.2 with 36.101 | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176614 | 3902 | - | Introduction of UE Transmit OFF power for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176616 | 3904 | - | Update of test configuration for Maximum input level for 4DL CA | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176618 | 3905 | - | Update of test configuration for ACS for 4DL CA | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176619 | 3906 | - | Update of test configuration for In-band blocking for 4DL CA | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176620 | 3907 | - | Update of test configuration for Out-band blocking for 4DL CA | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176621 | 3908 | - | Update of test configuration for Narrow band blocking for 4DL CA | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176622 | 3909 | - | Update of test configuration for Wideband intermodulation for 4DL CA | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176631 | 3912 | - | FDD Enhanced Downlink Control Channel Performance Type A for PHICH, 2 Tx Antenna Port under Asynchronous Network | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176632 | 3913 | - | FDD Enhanced Downlink Control Channel Performance Type A for PHICH, 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176634 | 3914 | - | FDD Enhanced Downlink Control Channel Performance Type B for PHICH, 2 Tx Antenna Port with Colliding CRS Dominant Interferer | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176635 | 3915 | - | FDD Enhanced Downlink Control Channel Performance Type B for PHICH, 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176636 | 3916 | - | TDD Enhanced Downlink Control Channel Performance Type A for PHICH, 2 Tx Antenna Port with Colliding CRS Dominant Interferer | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176638 | 3918 | - | TDD Enhanced Downlink Control Channel Performance Type B for PHICH, 2 Tx Antenna Port with Colliding CRS Dominant Interferer | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176639 | 3919 | - | TDD Enhanced Downlink Control Channel Performance Type B for PHICH, 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176640 | 3920 | - | FDD Enhanced Downlink Control Channel Performance Type A for EPDCCH, 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176642 | 3922 | - | TDD Enhanced Downlink Control Channel Performance Type A for EPDCCH, 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176643 | 3923 | - | FDD Enhanced Downlink Control Channel Performance Type A for EPDCCH, 2 Tx Antenna Port under Asynchronous Network | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176654 | 3925 | - | Correction to e-MTC TM9 PDSCH. | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176721 | 3937 | - | Missed additions and corrections in TS 36.521-1 | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176722 | 3938 | - | Correction to test requirements for B42 in TC 6.2.2A.2 | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176773 | 3940 | - | Alignment of 7.9A measurement frequency ranges with TS 36.101 | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176776 | 3941 | - | Correction of MPDCCH test cases | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176780 | 3942 | - | Corrections for NPDSCH-ConfigCommon-NB-DEFAULT in 6.3.4F.1 | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176792 | 3943 | - | Update of UL CA relative power control | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176796 | 3944 | - | Introduction of 4Rx CA TM9 demodulation test cases | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176827 | 3947 | - | Updates to Performance Test Cases for V2X Sidelink communications | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176839 | 3951 | - | Change of V2V TC names in TS 36.521-1 | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176840 | 3952 | - | Correction of eMTC chapter 8 TC | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176841 | 3953 | - | Editorial Change – Alignment of HST-SFN scenario and requirements to TS 36.101 | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176843 | 3955 | - | Editorial Change - Alignment of UE category M1 demodulation TCs to TS 36.101 | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176847 | 3957 | - | New feMTC test case, 7.3EC Reference sensitivity level for UE category M2 | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176848 | 3958 | - | New feMTC test case 6.2.2EB UE Maximum Output Power for UE category M2 | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176849 | 3959 | - | Correction 6.6.2.3F NB-IoT ACLR test conditions | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177092 | 3888 | 1 | Change of eMTC demodulation test cases numbering | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177094 | 3956 | 1 | Editorial Change - Alignment of Channel spacing for CA to 36.101 | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177112 | 3876 | 1 | Correction to test procedure for 6.3.5F.3 | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177113 | 3877 | 1 | Correction to test condition for 6.5.1F | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177114 | 3879 | 2 | Clean up on NB-IoT demodulation tests | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177117 | 3932 | 1 | eLAA: Updates to Maximum output power | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177300 | 3934 | 1 | Updates of the Receiver Test cases for 5DL CA | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177301 | 3936 | 1 | Updates of the Receiver Test cases for 4DL CA | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177303 | 3849 | 1 | New CA band combinations CA_1A-3A-40A, CA_1A-8A-40A, and CA_3A-8A-40A - Updates of 7.3A.5 REFSENS | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177304 | 3903 | 1 | Update of A-MPR and Additional spurious emissions for inter-band DL and UL CA for UL 64QAM | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177305 | 3911 | 1 | Update of test case 9.6.1.1_A.4 | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177306 | 3927 | 1 | Add a new test case 6.2.2A.4 to merge test cases 6.2.2A.4.1 and 6.2.2A.4.2 of UE Maximum Output Power for 3 UL CA | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177317 | 3917 | 1 | TDD Enhanced Downlink Control Channel Performance Type A for PHICH, 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177318 | 3921 | 1 | TDD Enhanced Downlink Control Channel Performance Type A for EPDCCH, 2 Tx Antenna Port with Colliding CRS Dominant Interferer | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177320 | 3926 | 1 | Correction to test case 6.2.2A.1 | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177321 | 3880 | 1 | Update 3DL CA Rx tests to support LAA band | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177323 | 3831 | 1 | Updated to RF section 8 & 9 test cases for missing TM9 in Test applicability | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177324 | 3884 | 1 | Correction to Test Parameters for Cat M1 PUCCH 1-0 static tests | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177325 | 3901 | 1 | Editorial correction to the operating bands for the vehicle mounted Band 30 UE | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177327 | 3930 | 1 | Correction to test case 9.3.2.1 | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177333 | 3834 | 1 | Update to existing FDD 4Rx SDR test cases | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177335 | 3837 | 1 | Addition of new 4Rx SDR test case - TDD single carrier | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177336 | 3835 | 1 | Addition of new 4Rx SDR test case - FDD 4DL | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177337 | 3836 | 1 | Addition of new 4Rx SDR test case - FDD 5DL | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177338 | 3838 | 1 | Addition of new 4Rx SDR test case - TDD 2DL | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177339 | 3839 | 1 | Addition of new 4Rx SDR test case - TDD 3DL | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177340 | 3840 | 1 | Addition of new 4Rx SDR test case - TDD 4DL | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177341 | 3841 | 1 | Addition of new 4Rx SDR test case - TDD 5DL | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177342 | 3842 | 1 | Addition of new 4Rx SDR test case - TDD FDD 2DL | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177343 | 3843 | 1 | Addition of new 4Rx SDR test case - TDD FDD 3DL | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177344 | 3845 | 1 | Addition of new 4Rx SDR test case - TDD FDD 5DL | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177349 | 3960 | - | Add a new test case 6.5.1A.4 to merge test cases 6.5.1A.4.1 and 6.5.1A.4.2 of Frequency error for CA (3UL CA) | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177352 | 3857 | 1 | Update of V2V RX Test cases | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177353 | 3794 | 1 | Update of TC7.6.2G.1 Out-of-band blocking for V2X Communication | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177354 | 3797 | 1 | Update of TC 7.8.1G.1 Wide band Intermodulation for V2X | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177355 | 3802 | 1 | Update of TC7.9G Spurious emissions for V2X | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177356 | 3808 | 1 | Introduction of new test case 6.3.5G.1 Power Control Absolute power tolerance for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177357 | 3858 | 1 | Introduction of Spurious response for V2V in 7.7G.1 | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177362 | 3859 | 1 | Update of V2X RX Test cases | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177363 | 3792 | 1 | Update to 5.2, 5.2G and 5.4.2G | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177364 | 3795 | 1 | Introduction of TC7.6.2G.2 Out-of-band blocking for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177365 | 3796 | 1 | Introduction of TC7.6.2G.3 Out-of-band blocking for V2X Communication / Intra-band contiguous multi-carrier operation | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177366 | 3798 | 1 | Introduction of TC 7.8.1G.2 Wide band Intermodulation for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177367 | 3799 | 1 | Introduction of TC 7.8.1G.3 Wide band Intermodulation for V2X Communication / Intra-band contiguous multi-carrier operation | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177368 | 3814 | 1 | Introduction of New TC 6.3.2G.3 Minimum output power for V2X Communication / Intra-band contiguous multi-carrier operation | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177369 | 3815 | 1 | Introduction of new test case 6.3.5G.2 Power Control Absolute power tolerance for V2X Communication / Intra-band contiguous multi-carrier operation | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177370 | 3861 | 1 | Introduction of Spurious response for V2X in 7.7G.3 | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177371 | 3946 | 1 | Addition of new TC for V2X_Demod | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177373 | 3816 | 1 | Introduction of numbering rules guideline for category M2 in Annex I | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177379 | 3847 | 1 | Correction to NB-IOT NPDSCH demod parameters | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177381 | 3939 | 1 | Correction in 8.12.2.1.2 to accommodate number of antennas | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177390 | 3825 | 1 | Updated clause 5.2E to include operating bands for UE category 1bis | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177391 | 3948 | 1 | Addition of new CAT1-bis TM9 Demod TCs | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177392 | 3949 | 1 | Cat1bis: Addition of new CAT1-bis TxDiv Demod TC | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177393 | 3950 | 1 | Cat1bis: Addition of new CAT1-bis SDR TCs | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177394 | 3954 | 1 | Addition of TC 7.3EB Reference Sensitivity for UE Category 1bis | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177395 | 3804 | 1 | Introduction of TC 8.9.1.1.2_1 FDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x1 for UE Category 1bis. | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177396 | 3805 | 1 | Introduction of TC 8.9.1.2.2_1 TDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x1 for UE Category 1bis | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177407 | 3833 | 1 | Introduction of new test case 6.3.5A.1.x Power Control Absolute power tolerance for CA (SRS carrier based switching) | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177408 | 3962 | 1 | Introduction of new test case TC 6.7A.4 "Transmit intermodulation for CA (3DL CA and 3UL CA) | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177418 | 3817 | 1 | Update to Rx test case 7.6.3A.8 and 7.8.1A.8 for 5DL CA | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177419 | 3854 | 1 | Update test parameters for TDD CQI Reporting under AWGN conditions – PUCCH 1-0 for CA (4DL CA) | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177420 | 3961 | 1 | Introduction of new test case TC 6.6.1A.4 “Occupied bandwidth for CA (3DL CA and 3UL CA) | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177421 | 3935 | 1 | Introduction of the new Test case 8.7.5.2_H.3 TDD-FDD 4DL CA SDR for 256QAM | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177423 | 3803 | 1 | Introduction of TC7.9F Spurious emissions for category NB1 | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177424 | 3868 | 1 | Update of 6.7F the transmit intermodulation for category NB1 | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177427 | 3855 | 1 | Introduction of TDD CQI Reporting under AWGN conditions – PUCCH 1-0 for CA (5DL CA) | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177428 | 3812 | 1 | Introduction of New TC 6.3.2G.1 Minimum output power for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177430 | 3899 | 1 | Adding new TC 6.7G.1 Transmit intermodulation for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177433 | 3844 | 1 | Addition of new 4Rx SDR test case - TDD FDD 4DL | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177434 | 3852 | 1 | Introduction of TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 with 256QAM for FDD PCell (2DL CA) | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177435 | 3853 | 1 | Introduction of TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 with 256QAM for TDD PCell (2DL CA) | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177436 | 3824 | 1 | Introduction of RF TC 8.9.2.1.1_1 | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177437 | 3813 | 1 | Introduction of New TC 6.3.2G.2 Minimum output power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177438 | 3856 | 1 | Introduction of Additional Maximum Power Reduction (A-MPR) for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177439 | 3890 | 1 | Introduction of In-band emissions for non-allocated RB for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions in 6.5.2.3G.2 | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177440 | 3900 | 1 | Adding new TC 6.7G.2 Transmit intermodulation for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177443 | 3821 | 1 | Optimisation for test cases 7.6.2A.x, statistical significance change. | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-177452 | 3924 | 1 | Clarification on the Order of SCCs | 14.4.0 | 14.5.0 |
| 2017-12 | RAN#78 | R5-176378 | 3848 | - | Band 72 introduction to 36.521-1 Chapter 5 | 14.5.0 | 15.0.0 |
| 2017-12 | RAN#78 | R5-176568 | 3898 | - | Band 72 introduction to 36.521-1 output power requirements | 14.5.0 | 15.0.0 |
| 2017-12 | RAN#78 | R5-177309 | 3830 | 1 | Align CA information in clause 5 with TS36.101 v15.0.0 | 14.5.0 | 15.0.0 |
| 2017-12 | RAN#78 | R5-177376 | 3826 | 1 | Added operating band and bandwidth for FDD Band 71 | 14.5.0 | 15.0.0 |
| 2017-12 | RAN#78 | R5-177441 | 3897 | 1 | Addition of LTE Band 71 into TS 36.521-1 | 14.5.0 | 15.0.0 |
| 2017-12 | RAN#78 | R5-177442 | 3931 | 1 | Addition of Band 71 in Receiver test cases | 14.5.0 | 15.0.0 |
| 2018-03 | RAN#79 | R5-180351 | 3965 | - | Band 68 introduction to 36.521-1 Chapter 5 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180356 | 3969 | - | Introduction of new V2X test case 6.3.5G.2 Power Control Absolute power tolerance for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180358 | 3971 | - | Update Annexes F.1.2 and F.3.2 for V2X test cases 6.3.5G.2 and 6.3.5G.3 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180385 | 3976 | - | Introduction of TC 9.7_1.1.2 TDD CQI reporting under AWGN conditions - PUCCH 1-0 for UE category 1bis | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180386 | 3977 | - | Introduction of TC 9.7_1.1.1 FDD CQI reporting under AWGN conditions - PUCCH 1-0 for UE category 1bis | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180387 | 3978 | - | Introduction of TC 9.7_1.1.3 FDD CQI reporting under fading conditions - PUSCH 3-0 for UE category 1bis | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180388 | 3979 | - | Introduction of TC 9.7_1.1.4 TDD CQI reporting under fading conditions - PUSCH 3-0 for UE category 1bis | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180389 | 3980 | - | Update of Annexes F.1.5 and F.3.5 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180414 | 3985 | - | Update of eIMTA performance test case 8.2.2.2.5 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180415 | 3986 | - | Addition of eIMTA CQI reporting test case 9.2.5 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180416 | 3987 | - | Addition of eIMTA CQI reporting test case 9.3.1.1.5 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180417 | 3988 | - | Addition of eIMTA CQI reporting test case 9.3.1.2.6 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180418 | 3989 | - | Add the test tolerance information into Annex F for multiple eIMTA test cases. | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180464 | 3991 | - | Update of TC8.13.3.6.1 and 8.13.3.6.2 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180466 | 3992 | - | Introduction of new test case 8.12.1.1.3 “Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone for NB2” | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180500 | 4008 | - | Band 72 introduction to 36.521-1 spurious requirements | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180509 | 4009 | - | Introduce definitions and symbols of NB2 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180513 | 4012 | - | Addition of Band 72 in 36.521-1 Receiver test cases | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180516 | 4013 | - | Addition of Band 72 in 36.521-1 Sections 9 and 12 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180575 | 4023 | - | Introduction of UE maximum output power for V2X Communication / Intra-band contiguous multi-carrier operation | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180576 | 4024 | - | Introduction of UE maximum output power for V2X Communication / Power class 2 / Non-concurrent with E-UTRA uplink transmissions | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180578 | 4025 | - | Introduction of UE Transmit OFF power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180739 | 4041 | - | Completion of FDD 4Rx SDR test cases | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180740 | 4042 | - | Completion of TDD 4Rx SDR test cases | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180741 | 4043 | - | Completion of TDD FDD 4Rx SDR test cases | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180742 | 4044 | - | Minimum conformance requirements update for spurious emission band UE co-existence for CA (editorial) | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180743 | 4045 | - | Introduction of Rel-15 test requirement for spurious emission band UE co-existence for CA | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180755 | 4049 | - | Added operation band and bandwidth information for Rel-15 CA | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180762 | 4051 | - | Resolve RMC name conflict in PHICH demodulation test | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180769 | 4052 | - | New feMTC test case 8.11.1.1.3.2, FDD and half-duplex FDD PDSCH 2 Tx Antenna Port supporting wideband transmission for UE category M2 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180770 | 4053 | - | New UL 256-QAM test case, 6.2.3_5, Maximum Power Reduction (MPR) for UL 256QAM | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180771 | 4054 | - | New UL 256-QAM test case, 6.6.2.3_5, Adjacent Channel Leakage power Ratio for UL 256QAM | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180772 | 4055 | - | Updates to legacy Tx test cases for UL 256-QAM | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180774 | 4056 | - | Update Annex A with reference measurement channels for UL 256-QAM | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180794 | 4059 | - | Editorial Correction – Alignment of Rel-14 CA configurations to TS 36.101 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180795 | 4060 | - | Editorial Correction – Alignment of RMC to 36.101 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180797 | 4062 | - | Correction of TC 7.3EB | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180798 | 4063 | - | Editorial Correction – Alignment of Rel-15 CA configuration to TS 36.101 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180799 | 4064 | - | Correction of A-MPR for Band 48 (NS_27) | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180812 | 4065 | - | Applicability of LTE DL Control Channel Interference Mitigation test cases | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180815 | 4066 | - | IM DLLCH Definitions, symbols and abbreviations | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180840 | 4067 | - | Update of Category M1 UL CEModeB TCs | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180843 | 4070 | - | Correction to LAA performance TCs | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180844 | 4071 | - | Correction to TC 6.7F | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180855 | 4078 | - | Introduction of General ON/OFF time mask for CA (3UL CA) | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180926 | 4089 | - | Editorial change of Additional spurious emissions for CA (inter-band DL CA and UL CA) for UL 64QAM | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180927 | 4090 | - | Update of TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (4DL CA) | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180928 | 4091 | - | Update of TDD Sustained data rate performance for CA (4DL CA) | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180929 | 4092 | - | Update of TDD FDD CA PDSCH Single Antenna Port Performance for TDD PCell (5DL CA) | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180935 | 4095 | - | Correction to Rx Test Environment for 4 and 5 DL CA with Wgap | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180938 | 4097 | - | Correction to NB-IoT Configured UE transmitted power test 6.2.5F | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180941 | 4098 | - | Correction to operating band for NB-IoT | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180942 | 4099 | - | Correction to test configuration tables for intra-contiguous 2CA Tx tests | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180943 | 4100 | - | Update to test requirements in CatM1 TC 6.7EA | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180947 | 4104 | - | Editorial correction to 4x4MIMIO demodulation test cases | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180949 | 4106 | - | Editorial correction in Annex F for CatM1 TC 8.11.1.1.1 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180992 | 4109 | - | Introduction of CA_3A-7A-20A (3DL-2UL) to section 7 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181055 | 4127 | - | Addition of the Band 74 information into TS 36.521-1 chapter 5 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181057 | 4129 | - | Addition of the Band 74 information into TS 36.521-1 chapter 7 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181080 | 4134 | - | Editorial Correction of DCI format in 4Rx demodulation test case | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181096 | 4140 | - | Introduction of 4Rx CA TDD TM9 demodulation test cases | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181097 | 4141 | - | Introduction of 4Rx CA FDD TM4 demodulation test cases | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181098 | 4142 | - | Introduction of 4Rx CA TDD TM4 demodulation test cases | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181099 | 4143 | - | Update of Annex F and G for new 4Rx CA demodulation test cases | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181102 | 4144 | - | Beamforming model for FD-MIMO | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181341 | 4068 | 1 | Updates of Annex A for LAA | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181342 | 4069 | 1 | Correction to LAA CQI TCs | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181361 | 4086 | 2 | Updates to PDSCH scheduling for CQI reporting test cases for CA | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181362 | 4103 | 1 | Correction to 4x4MIMO CSI test cases | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181511 | 4028 | 1 | Insert TT statement for V2V SEM and SE test cases in Annex F | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181516 | 4034 | 1 | Add TT statement for V2X SEM and SE test cases in Annex F | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181520 | 3972 | 1 | Updates of the CQI reporting TC 9.6.1.3.3 and 9.6.1.4.3 for TDD FDD CA | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181521 | 4076 | 1 | Introduction of Minimum Output Power for CA (3UL CA) | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181522 | 4077 | 1 | Introduction of UE Transmit OFF power for CA (3UL CA) | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181527 | 4096 | 1 | Editorial correction to Table 6.2.5.3-3 Delta TIB (three bands) | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181529 | 3974 | 1 | Band 68 introduction to 36.521-1 output power requirements | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181530 | 3975 | 1 | Band 68 introduction to 36.521-1 spurious requirements | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181531 | 4029 | 1 | Addition of Band 68 in 36.521-1 Receiver test cases | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181533 | 4083 | 1 | Update of CA Performance Test Cases | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181534 | 4115 | 1 | Correction to typo of the test case numbering of 8.2.2.1.1_A.3 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181537 | 4125 | 1 | Missing requirements for CA_29A-66A-66A, CA_29A-66C | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181538 | 4126 | 1 | Receiver requirements for CA_29A-66A-70A, CA_29A-66A-66A-70A, CA_29A-66A-66A-70C, CA_29A-66A-70C, CA_29A-66C-70A, CA_29A-66C-70C, CA_29A-70C, CA_66A-66A-70A, CA_66A-66A-70C, CA_66A-70A, CA_66A-70C, CA_66C-70A, CA_66C-70C | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181539 | 4040 | 1 | eLAA_Update to 6.3.5A.1.2 and 6.3.5A.2.2 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181540 | 4113 | 1 | Updates to test case 6.6.1A.2 Occupied bandwidth | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181543 | 4085 | 1 | Correction of test points and test requirement for TC 6.6.3.2A.2 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181544 | 4135 | 1 | Intra-band non-contiguous sensitivity test case corrections | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181545 | 4072 | 1 | Update of section 8.1.2 for DL 256QAM | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181546 | 4132 | 1 | CSI 4RX: Correction to RI tests and used reference channels and MCS schemes | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181547 | 4105 | 1 | Correction to LAA performance tests | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181549 | 4131 | 1 | Reference sensitivity adjustment due to support for 4 antenna ports | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181550 | 4133 | 1 | Correction of message exceptions with zeroTxPowerResourceConfigList-r10 and zeroTxPowerSubframeConfig-r10 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181558 | 4001 | 1 | Introduction of New TC 8.13.1.1.3 FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 with 256QAM (2DL CA) | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181559 | 4002 | 1 | Introduction of New TC 8.13.1.1.4 FDD PDSCH Closed Loop Four-Layer Spatial Multiplexing for CA (2DL CA) | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181561 | 4093 | 1 | Update of test applicability and minimum requirement in 9.6.1.3.4 and 9.6.1.4.4 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181563 | 3973 | 1 | Resubmission of Updates to Performance Test Cases for V2X Sidelink communications | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181564 | 3982 | 1 | Addition of new test case 6.5.2.1G.1, EVM for V2X non-concurrent with E-UTRA | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181565 | 3981 | 1 | Update of test case 6.5.1G.1, Frequency Error for V2X non-concurrent with E-UTRA | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181566 | 3983 | 1 | Addition of new test case 6.5.2.2G.1, Carrier leakage for V2X non-concurrent with E-UTRA | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181567 | 4014 | 1 | Introduction of Maximum Power Reduction (MPR) for V2X Communication / Power class 3 / Non-contiguous allocation of PSCCH and PSSCH / Non-concurrent with E-UTRA uplink transmissions | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181568 | 4019 | 1 | Update to General ON/OFF time mask for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181569 | 4021 | 1 | Introduction of PSSS/SSSS time mask for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181572 | 3990 | 1 | Editorial, Cleaning up the style of titles of V2X test cases in TS 36.521-1 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181573 | 4016 | 1 | Introduction of Configured UE transmitted Output Power for V2X Communication / Intra-band contiguous multi-carrier operation | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181574 | 4018 | 1 | Introduction of UE Transmit OFF power for V2X Communication / Intra-band contiguous multi-carrier operation | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181575 | 4020 | 1 | Introduction of General ON/OFF time mask for V2X Communication / Intra-band contiguous multi-carrier operation | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181577 | 4030 | 1 | Introduction of Spurious emission for V2X Communication / Sidelink simultaneous with E-UTRA uplink transmissions | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181578 | 4031 | 1 | Introduction of Spurious emission band UE co-existence for V2X Communication / Sidelink simultaneous with E-UTRA uplink transmissions | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181579 | 4032 | 1 | Introduction of Spectrum Emission Mask for V2X Communication /Sidelink simultaneous with E-UTRA uplink transmissions | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181580 | 4033 | 1 | Introduction of Additional Spectrum Emission Mask for V2X Communication / Sidelink simultaneous with E-UTRA uplink transmissions | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181581 | 4037 | 1 | Completion of Performance Test Case 14.8 for V2X Sidelink communications | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181582 | 4039 | 1 | Addition of new TC 6.6.1G.3 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181583 | 4094 | 1 | Addition of new TC 6.7G.3 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181602 | 4010 | 1 | Add the term NB2 into the applicability of minimum requirements | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181603 | 4011 | 1 | Add the term NB2 into Operating bands, Channel bandwidth and Carrier frequency | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181610 | 4128 | 1 | Addition of the Band 74 information into TS 36.521-1 chapter 6 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181615 | 4130 | 1 | Update of the operation bands about Category M2 and NB2 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181619 | 4138 | 1 | Introduction of CA_3A-7A-20A-32A 4DL/1UL to section 7 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181621 | 4082 | 1 | Update of 5DL CA Receiver Test Cases | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181622 | 4117 | 1 | Addition of the information about timing difference for TDD CA (2Rx) | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181624 | 4022 | 1 | Introduction of TDD sustained data rate performance for CA (5DL CA) | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181625 | 4047 | 1 | Updated to RF section 7 test cases for Rel-14 CA combination | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181626 | 4079 | 1 | Introduction of TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (5DL CA) | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181627 | 4080 | 1 | Introduction of TDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (5DL CA) | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181628 | 4081 | 1 | Introduction of TDD PDSCH Single Antenna Port Performance for CA (5DL CA) | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181629 | 4116 | 1 | Update test cases 6.2.2A.2 and 6.2.3A.2 for 12A-66A 2ULCA | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181631 | 4026 | 1 | Supplement the TC In-band emissions for non-allocated RB for V2X Communication/Non-concurrent with E-UTRA uplink transmissions with PSBCH testing | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181632 | 4027 | 1 | Update of the V2V SEM and SE test cases | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181633 | 4036 | 1 | Completion of Performance Test Cases 14.2 and 14.3 for V2V Sidelink communications | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181634 | 4107 | 1 | Update of Spectrum Emission Mask for V2X Communication / Non-concurrent with E-UTRA uplink transmissions in 6.5.2.4G.1 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181636 | 4035 | 1 | Addition of IBE and EVM test requirement for Elaa | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181639 | 4108 | 1 | eLAA: Updates to Maximum output power | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181641 | 4003 | 1 | Introduction of New TC 8.13.1.3.1 FDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model – Enhanced Performance Requirement Type A (2DL CA) | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181642 | 4004 | 1 | Introduction of New TC 8.13.1.4.1 FDD PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 Interference Model - Enhanced Performance Requirement Type A (2DL CA) | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181643 | 4005 | 1 | Introduction of New TC 8.13.2.3.1 TDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model – Enhanced Performance Requirement Type A (2DL CA) | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181644 | 4006 | 1 | Introduction of New TC 8.13.2.4.1 TDD PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 Interference Model - Enhanced Performance Requirement Type A (2DL CA) | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181645 | 3970 | 1 | Update Annexes F.1.2 and F.3.2 for 4.2GHz < f <= 6GHz of 6.2.2/6.3.2/6.3.3/6.3.5.1 and V2V Sidelink test case 6.3.5G.1 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181646 | 3967 | 1 | Update the message contents and test requirement for V2V Sidelink test case 6.3.5G.1 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181647 | 3968 | 1 | Update the message contents and test requirement for V2X test case 6.3.5G.3 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181648 | 4015 | 1 | Introduction of Maximum Power Reduction (MPR) for V2X Communication / Power class 3 / Intra-band contiguous multi-carrier operation | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181649 | 4058 | 1 | Update to UE Maximum Output Power for V2X Communication / Non-concurrent with E-UTRA uplink transmission | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181650 | 4048 | 1 | Align CA capability definitions for demodulation test cases in sub-clause 8.1.2.x with TS 36.101 v15.1.0 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181651 | 4050 | 1 | Updated to RF section 7 test cases for Rel-15 CA combination | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181653 | 4114 | 1 | Correction to test case 6.6.3.1A.1 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181654 | 4061 | 1 | Corrections for eMTC demodulation test cases | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181663 | 4007 | 1 | Update to Rx test cases for 5DL CA | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181664 | 4073 | 1 | Correction to 3DL/4DL CA Receiver Test Cases | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181361 | 4086 | 2 | Addition of missing CR "Updates to PDSCH scheduling for CQI reporting test cases for CA" | 15.1.0 | 15.1.1 |
| 2018-04 | RAN#79 | RP-180517 | 4145 | - | Resubmission of clause 8 part of CR: Cat1bis: Addition of new CAT1-bis SDR TCs Note: This is a resubmission of CR3950r1 which was not implemented after RAN#78. | 15.1.1 | 15.1.2 |
| 2018-06 | RAN#80 | R5-182229 | 4146 | - | Editorial change – Alignment of RMCs in HST TCs | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182253 | 4149 | - | Addition of TT for test case 8.2.1.9 in TS 36.521-1 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182254 | 4150 | - | Addition of TT for test case 8.2.2.9 in TS 36.521-1 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182273 | 4155 | - | Update minimum conformance requirements for B43 in Spurious emission band UE co-existence test case (editorial) | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182362 | 4163 | - | eLAA: RMC Update for eLAA | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182366 | 4164 | - | New test case: 7.4EC Maximum input level for UE category M2 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182414 | 4180 | - | Updating category M2 receiver test case 7.3EC with new bands (4, 14, 71) | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182531 | 4186 | - | Update to Transmit OFF power for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182532 | 4187 | - | Update to General ON/OFF time mask for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182534 | 4189 | - | Update to PSSS/SSSS time mask for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182539 | 4193 | - | Update to Maximum Power Reduction (MPR) for V2X Communication / Power class 3 / Contiguous allocation of PSCCH and PSSCH / Non-concurrent with E-UTRA uplink transmissions | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182590 | 4204 | - | Update to UE Maximum Output Power for V2X Communication / Non-concurrent with E-UTRA uplink transmission | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182591 | 4205 | - | Update to UE Maximum Output Power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182592 | 4206 | - | Update to UE Maximum Output Power for V2X Communication / Intra-band contiguous multi-carrier operation | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182602 | 4212 | - | Addition of Annex G for V2V/X performance requirements for Sidelink communications | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182606 | 4215 | - | Addition of V2V performance test 14.4 for Sidelink communications | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182626 | 4224 | - | Update to 6.6.1G.1 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182631 | 4227 | - | New LTE 3UL CA TC 6.6.3.2A.4 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182633 | 4228 | - | eLAA update to 6.3.5A.2.2 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182647 | 4230 | - | Resubmit the CR of TC 9.7_1.1.2 for UE category 1bis | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182661 | 4231 | - | Update of test case 6.5.2.1G.1, EVM for V2X non-concurrent with E-UTRA | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182662 | 4232 | - | Update of test case 6.5.2.2G.1, Carrier leakage for V2X non-concurrent with E-UTRA | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182663 | 4233 | - | Update of Annex E to add EVM calculation method for V2X communication | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182667 | 4235 | - | Addition of REFSENS for CA_40E | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182669 | 4236 | - | Addition of REFSENS for CA_41F | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182672 | 4237 | - | Introduction of band 73 to TS36.521-1 clause 5 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182687 | 4242 | - | Introduction of TC 6.3.5A.1.4 Power Control Absolute power tolerance for CA (3UL CA) | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182688 | 4243 | - | Introduction of TC 6.3.5A.2.4.1 Power Control Relative power tolerance for CA (intra-band contiguous 3DL CA and 3UL CA) | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182691 | 4246 | - | Introduction of TC 6.3.5A.2.4.3 Power Control Relative power tolerance for CA (intra-band non-contiguous 3DL CA and 3UL CA) | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182694 | 4247 | - | Introduction of TC 6.3.5A.3.4 Aggregate Power Control Tolerance for CA (3UL CA) | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182696 | 4249 | - | CA band combination CA_1A-18A-28A - Updates of test points for some Rx tests | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182699 | 4251 | - | CA band combination CA_3A-42A - Updates of 7.3A.6 REFSENS | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182702 | 4253 | - | CA band combination CA_41C-42C - Updates of 7.3A.9 REFSENS | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182704 | 4254 | - | Update of Editor’s note in clause 9.6.1.3.4 and 9.6.1.4.4 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182705 | 4255 | - | Introduction of TC 6.6.2.3A.4.3 Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous 3DL CA and 3UL CA) | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182709 | 4257 | - | CA band combinations CA_1A-3A-28A, CA_1A-3A-42A, CA_1A-42C and CA_3A-42C - Updates of 7.3A.5 REFSENS | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182714 | 4259 | - | New CA band combination CA_3A-18A - Updates of 7.3A.3 and 7.3A.6 REFSENS | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182715 | 4260 | - | Introduction of TC 6.6.3.1A.4.3 Transmitter Spurious emissions for CA (intra-band non-contiguous 3DL CA and 3UL CA) | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182722 | 4262 | - | CA band combinations CA_41A-42C and CA_41C-42A - Updates of 6.2.2A.4 MOP | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182725 | 4264 | - | CA band combinations CA_41A-42C and CA_41C-42A - Updates of 7.3A.5 REFSENS | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182727 | 4265 | - | Editorial Correction of SNR value for TC 8.10.2.1.1 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182733 | 4268 | - | Correction of REFSENS for CA_2A-4A-29A-30A and CA_4A-29A-30A | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182735 | 4270 | - | Correction of REFSENS for CA_4A-4A-12A and CA_4A-4A-13A | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182742 | 4271 | - | Editorial correction to 6.2.5A.3 test requirement | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182743 | 4272 | - | Correction to SRS time mask for UE category M1 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182746 | 4275 | - | Correction to eDL-MIMO RI Reporting Test | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182747 | 4276 | - | Correction of RMC name in PHICH demodulation test | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182748 | 4277 | - | Correction to TM4 4Tx Perf tests | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182750 | 4279 | - | Editorial correction to eMTC CSI tests | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182760 | 4280 | - | Updated RF clause 7.3A.0 for Rel-15 CA combination | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182771 | 4284 | - | Correction of MPR for Multi Cluster PUSCH test case 6.2.3_4 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182772 | 4285 | - | Correction of MPR for Multi Cluster PUSCH test case 6.2.3_2 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182804 | 4289 | - | Corrections to RMC for LAA performance TCs | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182806 | 4291 | - | Update of general sections of chapter 8 and 9 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182877 | 4304 | - | Clarification on the Order of SCCs | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182898 | 4305 | - | Update the minimum conformance requirements, message contents and test requirement for V2V Sidelink test case 6.3.5G.1 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182899 | 4306 | - | Correction to e-MTC CQI test cases | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182904 | 4307 | - | Introduction of new V2X test case 6.3.5G.2 Power Control Absolute power tolerance for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182906 | 4308 | - | Update the minimum conformance requirements, message contents and test requirement for V2X test case 6.3.5G.3 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182919 | 4312 | - | Introduction of 4Rx TDD FDD CA TM4 demodulation test cases | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182974 | 4316 | - | Addition of band 4, 14 and 71 for UE category M1 and NB1 to clause 6.6. | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182975 | 4317 | - | Addition of band 4, 14 and 71 for UE category M1 and NB1 to clause 7.3 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183002 | 4320 | - | Corrections for LAA performance test cases | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183005 | 4323 | - | Editorial correction of applicability and test rules for TDD-FDD CA configurations | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183006 | 4324 | - | Editorial correction to RMC for UL 256QAM | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183010 | 4325 | - | Alignment of test configuration tables for CA receiver test cases | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183254 | 4288 | 1 | Corrections to LAA performance TCs | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183278 | 4278 | 1 | Correction to LAA CSI reporting tests | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183713 | 4329 | - | Addition and correction of GNSS information for V2X for receiver tests | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183718 | 4188 | 1 | Test Tolerance and other details for CAT1-bis TM9 Demod TCs | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183731 | 4222 | 1 | Addition of new TC 7.4A.7_H | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183732 | 4223 | 1 | Addition of new TC 7.4A.8_H | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183735 | 4295 | 1 | Update of Name_Release mapping table for DL CA | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183744 | 4181 | 1 | LTE_B48_Reference sensitivity level correction | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183746 | 4203 | 1 | Correction for test requirement in TC 6.2.4_2 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183747 | 4273 | 1 | Editorial correction to additional spurious emission test | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183748 | 4287 | 1 | Correction to test points for NS_12 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183749 | 4274 | 1 | Correction to test configuration tables in intra-band non-contiguous CA Rx test cases | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183750 | 4300 | 1 | Decreasing test frequency for NB-IoT test | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183752 | 4151 | 1 | Correction to eMTC TM6 PDSCH applicability | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183754 | 4154 | 1 | Editorial: Adding titles to empty sub-clauses | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183760 | 4216 | 1 | Updates to Ch9 CA CQI Reporting Accuracy tests | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183761 | 4217 | 1 | Introduction of New TC 8.13.2.1.3 TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 with 256QAM (2DL CA) | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183762 | 4219 | 1 | Introduction of New TC 8.13.2.1.4 TDD PDSCH Closed Loop Four-Layer Spatial Multiplexing (2DL CA) | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183763 | 4221 | 1 | Introduction of New TC 8.13.3.7.1 TDD-FDD CA PDSCH Closed Loop Four-Layer Spatial Multiplexing for FDD PCell (2DL CA) | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183764 | 4225 | 1 | Introduction of New TC 8.13.3.7.2 TDD-FDD CA PDSCH Closed Loop Four-Layer Spatial Multiplexing for TDD PCell (2DL CA) | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183765 | 4267 | 1 | Editorial correction to 4 layer MIMO test | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183766 | 4290 | 1 | Update of RMCs for four antenna ports | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183770 | 4321 | 1 | Corrections of CA configurations | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183772 | 4147 | 1 | Align CA information in clause 5 with TS36.101 v15.2.0 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183773 | 4160 | 1 | Aligning CA delta TiB in sub-clause 6.2.5.3 with TS 36.101 v15.2.0 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183774 | 4161 | 1 | Aligning CA delta RiB in sub-clause 7.3.3 with TS 36.101 v15.2.0 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183776 | 4261 | 1 | CA band combinations CA_1A-3A-18A and CA_1A-3A-26A - Updates of 7.3A.5 REFSENS | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183779 | 4327 | 1 | Correction to Rx requirements in Band 70 CA configurations without Band 71 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183788 | 4332 | - | New CA band combinations CA_3A-7A-42A and CA_3A-20A-42A - Updates of 7.3A.5 REFSENS | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183789 | 4197 | 1 | Cleaning up TC 6.5.2.4G.1 and 6.6.3G.1 for V2V | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183790 | 4198 | 1 | Update of test requirement in TC 6.6.3G.1_1 for V2V | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183791 | 4199 | 1 | Introduction of TC 6.6.2.3G.1 for V2V | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183792 | 4208 | 1 | Correction to the V2V test applicability | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183793 | 4210 | 1 | Update to Additional Maximum Power Reduction (A-MPR) for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183794 | 4211 | 1 | Update to Configured UE transmitted Output Power for V2X Communication / Non-concurrent with E-UTRA uplink transmission | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183795 | 4213 | 1 | Completion of V2V performance tests for Sidelink communications | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183796 | 4226 | 1 | Update of test case 6.7G.1 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183797 | 4234 | 1 | Update of Annex F to add TT information for multiple V2V test cases | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183798 | 4310 | 1 | Update Annex F.3.2 for 4.2GHz < f <= 6GHz of 6.3.2/6.3.5.1 and V2V Sidelink test case 6.3.5G.1 according to R4-1805742 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183801 | 4330 | 1 | Addition and correction of GNSS information for V2X for transmitter tests – part 1 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183802 | 4331 | 1 | Addition and correction of GNSS information for V2X for transmitter tests – part 2 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183803 | 4168 | 1 | Addition and correction of GNSS information for V2X to clause 14 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183804 | 4169 | 1 | Deletion of GNSS information for V2X from annex C.2 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183805 | 4207 | 1 | Introduction of Maximum Power Reduction (MPR) for V2X Communication / Power class 2 / Contiguous allocation of PSCCH and PSSCH / Non-concurrent with E-UTRA uplink transmissions | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183806 | 4214 | 1 | Completion of V2X Performance TC 14.8 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183808 | 4309 | 1 | Introduction of new V2X test case 6.5.1G.2 Frequency error for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183809 | 4311 | 1 | Update Annex F.3.2 for V2X test cases 6.3.5G.2 and 6.3.5G.3 according to R4-1805742 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183813 | 4148 | 1 | Introduce HST-SFN into the Abbreviations of TS 36.521-1 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183814 | 4318 | 1 | Update of SRS switching test case 6.3.5A.1.5 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183815 | 4170 | 1 | Addition of new test case for Maximum input level for UE category 1Bis | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183816 | 4171 | 1 | LTE_New test case 7.5EB Adjacent Channel Selectivity (ACS) for UE category 1bis | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183817 | 4172 | 1 | LTE_New test case 7.6.1EB In-band blocking for UE category 1bis | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183818 | 4173 | 1 | LTE_New test case 7.6.2EB Out-of-band blocking for UE category 1bis | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183819 | 4174 | 1 | LTE_New test case 7.6.3EB Narrow band blocking for UE Category 1bis | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183820 | 4175 | 1 | LTE_New test case 7.7EB Spurious response for UE category 1bis | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183821 | 4176 | 1 | LTE_New test case 7.8.1EB Wide band Intermodulation for UE category 1bis | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183822 | 4177 | 1 | LTE_New test case 7.9EB Spurious emissions for UE category 1bis | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183823 | 4229 | 1 | Update of TC 8.9.1.1.2_1 FDD PDSCH Closed Loop Single Layer Spatial Multiplexing for UE category 1bis | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183824 | 4238 | 1 | Update of TC 8.9.1.2.2_1 TDD PDSCH Closed Loop Single Layer Spatial Multiplexing for UE category 1bis | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183825 | 4314 | 1 | Addition of new CAT1Bis demod TC 8.9.2.2.1_1 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183826 | 4326 | 1 | Addition of UE category 1bis transmit quality test cases | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183831 | 4281 | 1 | Updated general clause to include Cat-M2 information | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183832 | 4282 | 1 | Updated RF TC 8.11.2.x to cover Cat-M2 requirement | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183833 | 4283 | 1 | Added RMC information for category M2 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183835 | 4301 | 1 | Message Exceptions for LTE IM DLCCH EDPCCH Test Cases | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183836 | 4302 | 1 | Message Exceptions for LTE IM DLCCH PCFICH/PDCCH Test Cases | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183837 | 4303 | 1 | Message Exceptions for LTE IM DLCCH PHICH Test Cases | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183839 | 4156 | 1 | Correction to NB-IoT Configured UE transmitted power test 6.2.5F | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183840 | 4157 | 1 | Addition of Clause 6.2.5FA Configured UE transmitted Output Power for UE category NB1 and NB2 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183841 | 4158 | 1 | Addition of Clause 6.2.3FA MPR for category NB1 and NB2 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183842 | 4159 | 1 | Correction to Subclause 6.2.2F.3 and addition of Clause 6.2.2FA MOP for category NB1 and NB2 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183843 | 4162 | 1 | Correction to Clause 6.2.3F MPR for category NB1 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183853 | 4286 | 1 | Update of TCs 6.5.2A.1.2 and 6.5.2A.1.2_1 for eLAA | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183861 | 4200 | 1 | Addition of new UL 256QAM test case - MPR for multi cluster | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183862 | 4201 | 1 | Addition of new UL 256QAM test case - A-MPR | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183863 | 4202 | 1 | Addition of new UL 256QAM test cases - Annex F | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183868 | 4315 | 1 | Addition of band 4, 14 and 71 for UE category M1 and NB1 to clause 6.2 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183870 | 4218 | 1 | Addition of new TC 8.7.1.1_H.4 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183871 | 4245 | 1 | Introduction of TC 6.3.5A.2.4.2 Power Control Relative power tolerance for CA (inter-band 3DL CA and 3UL CA) | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183872 | 4250 | 1 | Introduction of TC 6.6.2.3A.4.1 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous 3DL CA and 3UL CA) | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183873 | 4252 | 1 | Introduction of TC 6.6.2.3A.4.2 Adjacent Channel Leakage power Ratio for CA (inter-band 3DL CA and 3UL CA) | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183874 | 4256 | 1 | Introduction of TC 6.6.3.1A.4.1 Transmitter Spurious emissions for CA (intra-band contiguous 3DL CA and 3UL CA) | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183875 | 4258 | 1 | Introduction of TC 6.6.3.1A.4.2 Transmitter Spurious emissions for CA (inter-band 3DL CA and 3UL CA) | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183876 | 4292 | 1 | Introduction of Maximum Power Reduction (MPR) for CA (3UL CA) | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183877 | 4178 | 1 | LTE_In Band Blocking test for 4DLCA combo CA_40E and 5DLCA-2ULCA combo CA_41F | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183878 | 4179 | 1 | LTE_Out_of_Band_Blocking for 4DLCA combo CA_40E and 5DLCA-2ULCA combo CA_41F | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183879 | 4195 | 1 | Update of TT in TC 6.5.2.3G.1 for V2V | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183880 | 4196 | 1 | Update of TT in TC 6.6.2.2G.1 for V2V | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183881 | 4209 | 1 | Update to Maximum Power Reduction (MPR) for V2X Communication / Power class 3 / Non-contiguous allocation of PSCCH and PSSCH / Non-concurrent with E-UTRA uplink transmissions | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183882 | 4220 | 1 | Addition of new TC 8.7.1.1_H.5 | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183884 | 4266 | 1 | Correction to test procedure for CQI reporting test cases for CA | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183885 | 4190 | 1 | Update to Maximum Power Reduction (MPR) for V2X Communication / Power class 3 / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183886 | 4191 | 1 | Introduction of Maximum Power Reduction (MPR) for V2X Communication / Power class 2 / Non-contiguous allocation of PSCCH and PSSCH / Non-concurrent with E-UTRA uplink transmissions | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183887 | 4192 | 1 | Update to Configured UE transmitted Output Power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183888 | 4194 | 1 | Introduction of General ON/OFF time mask for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183889 | 4182 | 1 | Introduction of New TC 6.5.2.1_2 Error Vector Magnitude (EVM) for UL 256QAM | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183890 | 4183 | 1 | Introduction of New TC 6.5.2A.1.1_2 EVM for CA (intra-band contiguous DL CA and UL CA) with UL 256QAM | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183891 | 4184 | 1 | Introduction of New TC 6.5.2A.1.2_2 Error Vector Magnitude (EVM) for CA (inter-band DL CA and UL CA) for UL 256QAM | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183892 | 4185 | 1 | Introduction of New TC 6.5.2A.1.3_2 Error Vector Magnitude (EVM) for CA (intra-band non-contiguous DL CA and UL CA) for UL 256QAM | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183897 | 4322 | 1 | Corrections to eMTC performance test cases | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183934 | 4334 | - | Update of V2V RF test cases for proper test mode setting | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183942 | 4293 | 2 | update of V2V RF TC | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183943 | 4333 | 1 | eLAA updates to Max power | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183944 | 4263 | 2 | Annex G update for NB-IOT demod test cases | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183945 | 4296 | 2 | Addition of new FDD SDR LAA test cases | 15.1.2 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183946 | 4298 | 2 | Addition of new TDD SDR LAA test cases | 15.1.2 | 15.2.0 |
| 2018-09 | RAN#81 | R5-184066 | 4337 | - | Resubmission of the Annexes of TC 6.5.2.3G.1 for V2V | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184099 | 4339 | - | Introduction of band 73 to UE maximum output power in TS36.521-1 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184130 | 4341 | - | Introduction of band 73 to Spurious emission in TS36.521-1 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184177 | 4342 | - | Updation of V2X TC 6.6.2.1G.2 & 6.6.2.1G.3 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184238 | 4355 | - | Correction to LAA CQI reporting tests | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184240 | 4357 | - | Correction to cqi-pmi-ConfigurationIndex for 5DL CA CQI Reporting tests | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184242 | 4359 | - | Correction to p-Max in configured UE transmitted output power tests for CA | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184243 | 4360 | - | Removal of the editor’s note regarding the faders in 4x4 CA demod tests | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184244 | 4361 | - | Editorial correction to inter-band UL CA tests | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184246 | 4363 | - | Editorial correction to test points for 8.7.5.1 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184247 | 4364 | - | Correction to 4CA and 5CA PDSCH demodulation tests | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184249 | 4366 | - | Editorial correction to Table 6.2.5.3-3 Delta TIB (three bands) | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184250 | 4367 | - | Correction to Test Tolerance for 4DL and 5DL Reference Sensitivity Test | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184252 | 4369 | - | Correction of connection diagram for 4Rx 3CC/4CC/5CC Rx tests | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184253 | 4370 | - | Clarify the definition of active time slot in out of band and spurious emission tests | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184254 | 4371 | - | Editorial correction to test points for CA_2A-12A-66A in 7.3A.5 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184285 | 4375 | - | Addition of REFSENS test configuration of CA_1A-3A-7A-8A | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184313 | 4376 | - | Demodulation test case 8.11.1.1.1 and 8.11.1.1.2 updated for category M2 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184316 | 4377 | - | Update of TDD-FDD CA PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model-Enhanced Performance Requirement Type A (2DL CA) | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184317 | 4378 | - | Update of TDD-FDD CA PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 Interference Model-Enhanced Performance Requirement Type A (2DL CA) | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184374 | 4382 | - | Update of TC 8.9.1.2.2_1 TDD PDSCH Closed Loop Single Layer Spatial Multiplexing for UE category 1bis | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184380 | 4383 | - | New feMTC test case 7.5EC Adjacent Channel Selectivity (ACS) for UE category M2 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184381 | 4384 | - | New feMTC test case 7.6.1EC In-band blocking for UE category M2 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184382 | 4385 | - | New feMTC test case 7.6.3EC Narrow band blocking for UE category M2 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184383 | 4386 | - | New feMTC test case 7.8.1EC Wide band intermodulation for UE category M2 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184384 | 4387 | - | Update of Annexes F.1.3 and F.3.3 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184385 | 4388 | - | Update of TC 6.6.2.3A.4 Adjacent Channel Leakage power Ratio for CA (3UL CA) | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184388 | 4389 | - | Update of TC 6.6.3.1A.4 Transmitter Spurious emissions for CA ( 3UL CA) | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184396 | 4391 | - | New CA band combination CA_8A-27A - Updates of 7.3A Refsens | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184412 | 4395 | - | Update to test case 6.5.2A.1.4 for 3UL CA | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184413 | 4396 | - | Update to test case 6.5.2A.2.4 for 3UL CA | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184414 | 4397 | - | Update to test case 6.5.2A.3.4 for 3UL CA | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184415 | 4398 | - | Update to test case 7.6.3A.8 and 7.8.1A.8 for 5DL CA | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184416 | 4399 | - | Cleanning up SDR test cases for 4CC and 5CC using UL RMC | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184417 | 4400 | - | Update to Performance test cases for LTE DL CA 4 Rx antenna ports | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184539 | 4410 | - | Update of DL256QAM RF test cases 7.4A.7_H,7.4A.8_H,8.7.1.1_H.4 and 8.7.1.1_H.5 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184540 | 4411 | - | Update MU and TT of V2X test case 6.5.1G.2 Frequency error for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184541 | 4412 | - | Correction to eMTC tc 6.2.2EA, UE Maximum Output Power for UE category M1 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184557 | 4420 | - | Addition of new UL 256QAM test case - A-MPR for 2UL CA inter-band | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184558 | 4421 | - | Addition of new UL 256QAM test case - Annex F | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184559 | 4422 | - | Updates to feMTC tc 6.2.2EC UE Maximum Output Power for UE category M2 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184585 | 4423 | - | Updated test requirement to RF clause 7 test cases for CA band | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184586 | 4424 | - | Align CA information in clause 5 with TS36.101 v15.3.0 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184604 | 4427 | - | Introduction of new test case TC 6.6.2.2A.4 Additional Spectrum Emission Mask for CA (3UL CA) | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184639 | 4430 | - | CA_1A-3A-26A(1UL) –Updates of 7.3A.5 REFSENS | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184644 | 4433 | - | Editorial: Cleaning up the style of Annex B.4.1A in TS 36.521-1 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184647 | 4435 | - | Correction of test requirements in 7.3.5A REFSENSE | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184650 | 4437 | - | Editorial correction of Table 7.6.3A.5.4.1-1 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184651 | 4438 | - | Update of TC8.12.1.1.3 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184660 | 4444 | - | Update to Maximum Power Reduction (MPR) for V2X Communication / Power class 3 / Contiguous allocation of PSCCH and PSSCH / Non-concurrent with E-UTRA uplink transmissions | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184695 | 4448 | - | Resubmission of update of TC8.13.3.6.1 and 8.13.3.6.2 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184728 | 4451 | - | Update of TC7.6.2G.1 Out-of-band blocking for V2X Communication | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184729 | 4452 | - | Update of TC 7.8.1G.1 Wide band Intermodulation for V2X | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184733 | 4453 | - | Update of TC7.9G Spurious emissions for V2X | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184762 | 4455 | - | Aligning CA delta TiB in sub-clause 6.2.5.3 with TS 36.101 v15.3.0 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184764 | 4456 | - | Aligning CA delta RiB in sub-clause 7.3.3 with TS 36.101 v15.3.0 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184797 | 4459 | - | Clarification for chapter 8 & 9 general sections | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184811 | 4462 | - | Correction to Note 1 of Table 7.3A.0-0bB Reference sensitivity level for CA | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184812 | 4463 | - | Correction to Power Control for CA Test Procedure of TCs 6.3.5A.1.1, 6.3.5A.1.2 and 6.3.5A.1.4 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184818 | 4464 | - | Correction to Message Content for TC 6.2.4EA A-MPR for UE Category M1 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184832 | 4465 | - | Addition of Test point and test requirements for CA_3A-41A, 41A-42A,3A-42A spurious test cases | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184839 | 4467 | - | Update of TDD PDSCH Closed Loop Multi Layer Multiplexing 4x2 for CA (5DL CA) | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184840 | 4468 | - | Update of TX Minimum Ouput Power for CA (3UL CA) | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184842 | 4470 | - | Introduction of Maximum Power Reduction (MPR) for CA (3UL CA) | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184844 | 4472 | - | Addition of new 2CA band combination for CA REFSENS | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184886 | 4483 | - | LTE_New test case 6.2.2EB UE Maximum Output Power for UE category 1bis | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184935 | 4485 | - | Cat1bis new RF test case: Occupied bandwidth for UE category 1bis | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184936 | 4486 | - | Cat1bis new RF test case: Spectrum Emission Mask for UE category 1bis | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184937 | 4487 | - | Cat1bis new RF test case: Additional Spectrum Emission Mask for UE category 1bis | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184938 | 4488 | - | Cat1bis new RF test case: Adjacent Channel Leakage power Ratio for UE category 1bis | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184939 | 4489 | - | Cat1bis new RF test case: Spurious emission for UE category 1bis | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184941 | 4491 | - | Editorial correction of NS requirements | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184991 | 4494 | - | Correction to Reference sensitivity level for CA UL Allocation in Table 7.3A.1.4.1-1 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185174 | 4457 | 1 | Reference sensitivity requirements for CA_66A-66A-70C-71A, CA_66A-66A-70A-71A, CA_66A-70C-71A, CA_66A-70A-71A, CA_66A-66A-71A, CA_70A-71A, CA_66A-71A, CA_66C-70C-71A, CA_66C-70A-71A, CA_70C-71A, CA_66C-71A | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185175 | 4492 | 1 | Update 3UL CA test cases 6.2.2A.4 and 6.5.1A.4 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185176 | 4493 | 1 | Update to test configuration tables and requirement tables for 12A-66A 2ULCA test cases. | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185185 | 4380 | 2 | Band 41 HPUE Maximum Output Power | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185186 | 4484 | 1 | 4Rx support in some 4DL CA and 5DL CA Demodulation test cases | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185330 | 4497 | - | Addition of test points for UL-CA_4A-12A in TC 6.3.2A.2 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185331 | 4498 | - | Addition of test points for UL-CA_4A-12A in TC 6.3.5A.2.2 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185343 | 4340 | 1 | New CA band combination CA_1A-3A-7A-20A - Update of table 7.3A.5, 7.3A.9 and refsens | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185354 | 4346 | 1 | Update of V2V TCs 6.5.2.3G.1 & 6.5.2.4G & 6.6.2.2G.1& 6.6.3G.1 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185355 | 4401 | 1 | Update to test case 6.3.2G.1 for V2V services based on LTE sidelink | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185356 | 4439 | 1 | Update of Additional Maximum Power Reduction (A-MPR) for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185357 | 4445 | 1 | Update to Annex G for V2V/V2X Demod Perf tests | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185358 | 4454 | 1 | Update of multiple V2V RF test cases for proper test mode setting | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185360 | 4348 | 1 | Update of V2X TCs 6.6.3G.2_1 & 6.6.3G.3_1 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185361 | 4349 | 1 | Update of V2X TCs 6.6.3G.2 & 6.6.3G.3 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185362 | 4350 | 1 | Correction to 36521-1-chapter 6_V2V and V2X | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185363 | 4372 | 1 | Cleaning up V2X test cases in TS 36.521-1 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185364 | 4402 | 1 | Update to test case 6.3.2G.2 for LTE-based V2X Services | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185365 | 4403 | 1 | Update to test case 6.3.2G.3 for LTE-based V2X Services | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185366 | 4413 | 1 | Update of V2X test case 6.7G.2 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185367 | 4414 | 1 | Addition of new TC 6.6.2.3G.2 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185368 | 4415 | 1 | Introduction of new V2X test case 6.5.2.1G.2 Error Vector Magnitude (EVM) for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185369 | 4416 | 1 | Introduction of new V2X test case 6.5.2.1G.3 Error Vector Magnitude (EVM) for V2X Communication / Intra-band contiguous multi-carrier operation | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185370 | 4440 | 1 | Update to Additional Maximum Power Reduction (A-MPR) for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185371 | 4458 | 1 | Updates to Align Initial Conditions with Other V2X Test Cases | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185372 | 4474 | 1 | Update of V2X Receiver Test Cases | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185375 | 4356 | 1 | Correction to 4x4 CA demodulation tests | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185377 | 4475 | 1 | TS 36.521-1 Annex F update for UE cat 1bis test cases | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185378 | 4476 | 1 | LTE_New test case LTE_New test case 6.3.4EB ONOFF time mask for UE category 1bis | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185379 | 4477 | 1 | LTE_New test case 6.3.5EB Power Control for UE category 1bis | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185380 | 4479 | 1 | LTE_New test case 6.3.2EB Minimum Output Power for UE category 1bis | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185381 | 4481 | 1 | LTE_New test case 6.2.4EB Additional Maximum Power Reduction (A-MPR) for UE category 1bis | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185382 | 4478 | 1 | LTE_New test case 6.3.3EB UE Transmit OFF power for UE category 1bis | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185383 | 4480 | 1 | LTE_New test case 6.2.5EB Configured UE transmitted Power for UE category 1bis | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185384 | 4482 | 1 | LTE_New test case 6.2.3EB Maximum Power Reduction (MPR) for UE category 1bis | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185385 | 4490 | 1 | Cat1bis new RF test case: Transmit intermodulation for UE category 1bis | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185387 | 4434 | 1 | Introduction of FDD PDSCH Single-layer Spatial Multiplexing for FD-MIMO | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185388 | 4436 | 1 | Introduction of TDD PDSCH Single-layer Spatial Multiplexing for FD-MIMO | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185406 | 4431 | 1 | Introduction of Additional spurious emissions for UL 256QAM | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185426 | 4379 | 1 | New test case: 6.2.3A.3_2, Maximum Power Reduction (MPR) for CA (intra-band non-contiguous DL CA and UL CA) for UL 256QAM | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185427 | 4449 | 1 | New UL 256QAM test case: 6.6.2.2_2 Additional Spectrum Emission Mask for UL 256QAM | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185431 | 4351 | 1 | eLAA_correction to 6.3.5A.1.2 and 6.3.5A.2.2 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185432 | 4353 | 1 | eLAA_Update to 6.6.3.1A.2 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185433 | 4447 | 1 | eLAA: Updates to Adjacent Channel Leakage power Ratio for CA | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185434 | 4419 | 1 | Addition of new UL 256QAM test case - A-MPR for 2UL CA intra-band contiguous | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185435 | 4418 | 1 | Correction of test points and test requirements for TC 6.2.4A.2_1 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185436 | 4417 | 1 | Correction of test points and test requirements for TC 6.2.4A.2 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185438 | 4343 | 1 | Introduction of V2X TC 6.5.2.4G.3 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185439 | 4344 | 1 | Updation of V2X TC 6.6.2.2G.2 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185440 | 4345 | 1 | Updation of V2X TCs 6.5.2.3G.2 and 6.5.2.3G.3 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185441 | 4347 | 1 | TT statements for V2X TCs | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185448 | 4461 | 1 | Corrections to A-MPR values for NS_27 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185450 | 4358 | 1 | Correction to message exception for 8.2.1.4.2_A.4 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185451 | 4362 | 1 | Editorial correction to RMC name in PHICH demodulation test | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185452 | 4365 | 1 | Corrections to CA Test Configuration Table in section 7 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185493 | 4373 | 1 | Correction of test configuration of CA_20A-42A REFSENS | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185494 | 4374 | 1 | Update of 7.3A.5 REFSENS for Rel-13 CA configurations | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185497 | 4392 | 1 | New test case 6.3.5FA.1 Power Control Absolute power tolerance for category NB1 and NB2/Power Class 6 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185498 | 4393 | 1 | New test case 6.3.5FA.2 Power Control Relative power tolerance for category NB1 and NB2/Power Class 6 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185499 | 4394 | 1 | New test case 6.3.5FA.3 Aggregate power control tolerance for category NB1 and NB2/Power Class 6 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185506 | 4473 | 1 | Addition of new 3CA band combination for CA REFSENS | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185508 | 4469 | 1 | Introduction of Additional Maximum Power Reduction (A-MPR) for CA (3UL CA) | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185509 | 4471 | 1 | Introduction of General ON/OFF time mask for CA (3UL CA) | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185514 | 4408 | 1 | Updates of receiver test cases for CA_XA-YE | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185515 | 4409 | 1 | Updates of receiver test cases for CA_XA-YD | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185517 | 4390 | 1 | Updates of Band 74 information about Tx test cases in TS 36.521-1 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185522 | 4338 | 1 | Update Clause in 6.2.5EA Configured UE transmitted Power for UE category M1 | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185530 | 4446 | 1 | Clarification on frame structure in 4Rx performance and CSI test principles sections | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185535 | 4443 | 1 | Update to General ON/OFF time mask for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185538 | 4425 | 1 | Introduction of new test case TC 6.2.5A.4 Configured UE transmitted Output Power for CA (3UL CA) | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185539 | 4426 | 1 | Introduction of new test case TC 6.6.2.1A.4 Spectrum emission mask for CA (3UL CA) | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185540 | 4429 | 1 | Update of test case TC 6.7A.4 "Transmit intermodulation for CA (3DL CA and 3UL CA)" | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185542 | 4432 | 1 | CA_3A-18A - Updates of some Tx test cases | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185185 | 4380 | 2 | fix wrong change of R5-185185 | 15.3.0 | 15.3.1 |
| 2018-12 | RAN#82 | R5-186580 | 4528 | - | Update NB-IOT Spurious Emission UE co-existence | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-186582 | 4530 | - | Update the 6.2.3FA to reduce test points | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-186583 | 4531 | - | Update Rel-14 NB-IOT Tx test cases to support category NB2 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-186584 | 4532 | - | Update Rel-14 NB-IOT Rx test cases to support category NB2 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-186588 | 4533 | - | Update 8.1.2.2 Definition of CA capability for 4DL and 5DL CA | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-186589 | 4534 | - | Update 4DL and 5DL performance test cases (Editorial) | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-186590 | 4535 | - | Addition of Statistic information for 4DL and 5DL test cases | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-186693 | 4538 | - | Update of V2X EVM TCs | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187004 | 4540 | - | Update 6.2.4G.1 the V2V AMPR test case to remove brackets and add Power Class 2 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187049 | 4541 | - | Editorial correction to 4CA and 5CA ACS test cases | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187050 | 4542 | - | Editorial correction to 5DL CA blocking tests | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187054 | 4546 | - | Editorial correction to 6.6.3.1A.2 Transmitter Spurious emissions for CA | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187056 | 4548 | - | Correction to 6.2.5A.3 Configured UE transmitted Output Power for CA | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187060 | 4550 | - | Correction to 4x4 CA demodulation tests | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187062 | 4551 | - | Correction to Minimum Test time for 8.12.1.1.2 Demodulation of NPDSCH | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187065 | 4552 | - | Removal of TC 7.3F.2 Reference sensitivity level with repetition for Category NB1 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187066 | 4553 | - | Correction to LAA CQI tests | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187068 | 4554 | - | Correction to 6.6.3.3A.2 Additional spurious emissions for CA | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187069 | 4555 | - | Correction to 6.2.4A.2 Additional Maximum Power Reduction (A-MPR) for CA | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187072 | 4558 | - | Correction to 2x4 CA demodulation tests | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187092 | 4569 | - | Introduction of New test case 6.6.2.2A.1_2 Additional Spectrum Emission Mask for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187093 | 4570 | - | Introduction of New test case 6.6.2.2A.2_2 Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA) for UL 256QAM | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187094 | 4571 | - | Update to Tx test cases for UL 256QAM | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187096 | 4572 | - | Update to 3 Tx test case for 3UL CA | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187143 | 4580 | - | Align CA information in clause 5 with TS36.101 v15.4.0 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187145 | 4582 | - | Updated test requirement to RF clause 7 test cases for CA band | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187147 | 4584 | - | Correction to test configuration table for CA band combination | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187154 | 4585 | - | Added applicability of requirements for UEs supporting coverage enhancement | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187182 | 4586 | - | Clarification for Cat-M1 Reference Sensitivity Test Requirement | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187192 | 4590 | - | Change of test point applicability in TC 6.2.4_2 due to 256QAM support | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187193 | 4591 | - | Change of test point applicability in TC 6.6.2.2_1 due to 256QAM support | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187195 | 4592 | - | Change of test point applicability in TC 6.6.2.3_4 due to 256QAM support | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187197 | 4594 | - | Change of test point applicability in TC 6.6.3.3_1 due to 256QAM support | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187202 | 4596 | - | Addition of new UL 256QAM test case - MPR for intra-band contiguous | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187205 | 4598 | - | Addition of new UL 256QAM test case - ACLR for multi cluster | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187210 | 4600 | - | Addition of new UL 256QAM test case - ALCR for intra-band contiguous | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187212 | 4602 | - | Addition of new UL 256QAM test case - ACLR for inter-band | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187291 | 4603 | - | CA_3A-28A - Updates of some Tx test cases | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187293 | 4604 | - | CA_11A-18A - Updates of some Tx test cases | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187295 | 4605 | - | CA_3A-41A-42C(1UL) - Updates of 7.3A.9 REFSENS | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187298 | 4606 | - | CA_3A-41C-42A(1UL) - Updates of 7.3A.9 REFSENS | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187300 | 4607 | - | CA_3A-41C-42C(1UL) - Updates of 7.3A.10 REFSENS | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187322 | 4609 | - | Correction to clause 9.1.1.4.2 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187329 | 4611 | - | Introduction of Additional spurious emissions for UL 256QAM | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187335 | 4614 | - | Test points and test requirements in TC 7.3A.9 for CA_2A-66C-71A and CA_2C-66A-66A | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187340 | 4615 | - | Introduction of CA_2A-66C-71A in Rx test cases | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187343 | 4616 | - | Correction of applicability statement in TC 6.2.4A.1 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187344 | 4617 | - | Addition of new Rel-14 CA configurations into REFSENS test | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187345 | 4618 | - | Addition of new Rel-15 CA configurations into REFSENS test | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187346 | 4619 | - | Correction of applicability in TC 6.6.2.2A.2 and TC.6.6.3.3A.2 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187347 | 4620 | - | Correction for test procedure step 4 in TC 6.6.3.3A.1 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187349 | 4622 | - | Correction for test procedure in TC 6.6.3.3A.1_1 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187350 | 4623 | - | Correction for test procedure in TC 6.6.3.3A.2_1 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187363 | 4626 | - | Corrections to TC 7.3A.5 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187364 | 4627 | - | Correction to general clause 7.1 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187365 | 4628 | - | Corrections to eMTC CQI Test Cases | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187389 | 4630 | - | Removing editor’s note in FDD 5DL CA test cases | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187473 | 4631 | - | MU for UL 256QAM EVM test cases updated | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187476 | 4632 | - | Introduction of FD-MIMO test case: FDD PMI Reporting with 12Tx Class A codebook – PUSCH 3-1 (Single PMI) for FD-MIMO | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187535 | 4637 | - | Update the parameters to test cases 7.6.1A.1, 7.6.1A.2, 7.6.1A.3 and 7.6.1A.4 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187541 | 4640 | - | Update test case to cover band 46 for test cases 7.5A.7 and 7.5A.8 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187861 | 4629 | 1 | Update of V2X RF Receiver Test Cases for Intra-band contiguous | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187862 | 4560 | 1 | Editorial: 6.2.2EC, UE Maximum Output Power for UE category M2 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187871 | 4638 | 1 | Update test case to cover band 46 for test cases 7.6.1A.7 and 7.6.1A.8 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187872 | 4639 | 1 | Update to test cases 7.6.1A.5 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187873 | 4561 | 1 | Introduction of tc 6.2.3EC, Maximum Power Reduction (MPR) for UE category M2 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187874 | 4562 | 1 | Introduction of tc 6.2.5EC Configured UE transmitted Power for UE category M2 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187875 | 4563 | 1 | Introduction of tc 6.3.2EC, Minimum Output Power for UE category M2 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187876 | 4564 | 1 | Introduction of tc 6.3.3EC, UE Transmit OFF power for UE category M2 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187877 | 4565 | 1 | Introduction of 6.3.4EC.1, General ON/OFF time mask for UE category M2 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187878 | 4566 | 1 | Introduction of 6.3.5EC.1, Power Control Absolute power tolerance for UE category M2 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187879 | 4567 | 1 | Introduction of 6.3.5EC.2, Power Control Relative power tolerance for UE category M2 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187880 | 4568 | 1 | Introduction of 6.3.5EC.3, Aggregate power control tolerance for UE category M2 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187916 | 4547 | 1 | Editorial correction to 6.5.1A.2 Frequency error for CA | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187917 | 4556 | 1 | Correction to 6.2.4A.2_1 and 6.6.2.2A.2_1 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187918 | 4543 | 1 | Correction to the test configuration table of 7.3A.6 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187940 | 4499 | 1 | Addition of new Test Configuration on TC 6.2.2A.2 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187941 | 4500 | 1 | Addition of new Test Configuration on TC 6.2.3A.2 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187942 | 4501 | 1 | Addition of new Test Configuration on TC 6.2.3A.2_1 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187943 | 4502 | 1 | Addition of new Test Configuration on TC 6.2.4A.2 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187944 | 4504 | 1 | Addition of new Test Configuration on TC 6.2.5A.3 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187945 | 4505 | 1 | Addition of new Test Configuration on TC 6.3.4A.1.2 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187946 | 4506 | 1 | Addition of new Test Configuration on TC 6.3.5A.1.2 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187947 | 4507 | 1 | Addition of new Test Configuration on TC 6.3.5A.3.2 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187948 | 4509 | 1 | Addition of new Test Configuration on TC 6.5.2A.1.2 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187949 | 4510 | 1 | Addition of new Test Configuration on TC 6.5.2A.1.2_1 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187950 | 4511 | 1 | Addition of new Test Configuration on TC 6.5.2A.2.2 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187951 | 4512 | 1 | Addition of new Test Configuration on TC 6.5.2A.3.2 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187952 | 4513 | 1 | Addition of new Test Configuration on TC 6.6.1A.2 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187953 | 4514 | 1 | Addition of new Test Configuration on TC 6.6.2.1A.2 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187954 | 4515 | 1 | Addition of new Test Configuration on TC 6.6.2.2A.2_1 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187955 | 4517 | 1 | Addition of new Test Configuration on TC 6.6.2.3A.2 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187956 | 4518 | 1 | Addition of new Test Configuration on TC 6.6.2.3A.2_1 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187957 | 4519 | 1 | Addition of new Test Configuration on TC 6.6.3.1A.2 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187958 | 4520 | 1 | Addition of new Test Configuration on TC 6.7A.2 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187963 | 4529 | 1 | Update the general sections for Rel-14 NB-IOT (Editorial) | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187965 | 4593 | 1 | Aligning CA delta TiB in sub-clause 6.2.5.3 with TS 36.101 v15.4.0 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187966 | 4595 | 1 | Aligning CA delta RiB in sub-clause 7.3.3 with TS 36.101 v15.4.0 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187967 | 4624 | 1 | Correction in TC 7.3A.3 for CA configuration CA_2A-71A | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187968 | 4625 | 1 | Correction in TC 7.3A.5 for CA configuration CA_2A-66A-71A | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187970 | 4587 | 1 | Change of applicability in UL 64QAM MPR and A-MPR tests due to UL 256QAM support | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187971 | 4588 | 1 | Change of applicability in UL 64QAM A-SEM and ACLR and Additional spurious tests due to UL 256QAM support | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187972 | 4597 | 1 | Addition of new UL 256QAM test case - MPR for inter-band | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187973 | 4612 | 1 | Addition of new UL 256QAM test case - Annex F | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187974 | 4537 | 1 | Introduction of Power Class 1 for B31 and B72 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187977 | 4536 | 1 | Introduction of the support of ProSe for B72 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187979 | 4521 | 1 | Correction to TC 7.7B Spurious response for UL-MIMO | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187980 | 4635 | 1 | Updating test case 6.2.4A.2 Additional Maximum Power Reduction (A-MPR) for CA (inter-band DL CA and UL CA) | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187984 | 4579 | 1 | Updated RF clause 9.8 to cover Cat-M2 requirement | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187988 | 4581 | 1 | Align CA information in clause 7.3A.0 with TS36.101 v15.4.0 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187990 | 4610 | 1 | Update of test case 9.6.1.1_A.4 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187991 | 4539 | 1 | Alignment of 256QAM EVM Test Tolerance between LTE and NR | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187992 | 4634 | 1 | Updating test case 6.6.2.2 Additional spectrum emission mask and test case 6.6.3.3 Additional spurious emission | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-188040 | 4608 | 1 | Correction of RB allocation for 3DL_CA_1A-3A-41A | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-188051 | 4559 | 1 | Correction to the MU for UL CA | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-188066 | 4573 | 2 | Update TC 6.6.3G.2_1 | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-188233 | 4633 | 1 | Introduction of FD-MIMO test cases in Annexes | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-188234 | 4574 | 1 | Maximum Output Power for CA | 15.3.1 | 15.4.0 |
| 2018-12 | RAN#82 | R5-188235 | 4641 | 1 | MPR for CA (intra-band contiguous DL CA and UL CA) | 15.3.1 | 15.4.0 |
| 2019-03 | RAN#83 | R5-191106 | 4645 | - | Incorrect numbering in minimum conformance requirements of TC 6.6.1 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191107 | 4646 | - | Missing message contents exceptions in TC 6.2.4EA | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191114 | 4653 | - | Addition of new UL feMTC test cases - Annex F | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191146 | 4654 | - | Incorrect numbering in minimum conformance requirements of TC 6.6.1EA | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191158 | 4655 | - | Table format correction to E-UTRA operating bands table | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191171 | 4658 | - | Update of TCs 6.6.3G.2 & 6.6.3G.2_1 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191173 | 4660 | - | Addition of TT statement for TCs 6.6.3G.2 & 6.6.3G.2_1 & 6.6.3G.2_2 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191232 | 4661 | - | Test tolerance and MU update for UE category 1bis transmit signal quality test cases | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191238 | 4662 | - | Editorial Correction of UE Category 1bis Rx test cases | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191239 | 4663 | - | Editorial Correction of title of TC 6.5.2.1EB | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191278 | 4665 | - | Editorial Correction - Alignment of CQI Index to MCS Mapping to TS 36.101 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191282 | 4666 | - | Adding test case 6.6.2.3A.3_2, Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous DL CA and UL CA) for UL 256QAM | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191765 | 4690 | - | Introduction of Additional spurious emissions for UL 256QAM | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191772 | 4691 | - | Correction to 6.3.3F test tolerance | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191773 | 4692 | - | Correction to the test procedure of 6.5.2A.3.2 In-band emission with inter-band UL CA | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191774 | 4693 | - | Correction to connection diagram for 6.6.3F.1 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191775 | 4694 | - | Correction to the note in delta RIB table | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191776 | 4695 | - | Update 4DL CA Rx tests to support LAA band | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191778 | 4697 | - | Correction to 3DL CA Name/Release mapping table | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191779 | 4698 | - | Correction to UL RB number for NS_27 in A-MPR test | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191780 | 4699 | - | Correction to test requirements in A-MPR for intra-band contiguous UL CA | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191782 | 4701 | - | Correction to test configuration for CA_3A-28A in 6.6.3.2A.2 Spurious emission band UE co-existence for CA | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191783 | 4702 | - | Adding test configuration for CA_1A-3A-7A-20A in 7.3A.9 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191785 | 4704 | - | Editorial correction to 6.3.4A.1.4 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191786 | 4705 | - | Editorial correction to demodulation tests for 4Rx CA | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191787 | 4706 | - | Core alignment of RMC table for TM9 CSI-RS | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191870 | 4711 | - | RB_correction in 6.6.3G.1 of TS36.521-1 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191976 | 4717 | - | Introduction of band 73 to Reference sensitivity level in TS36.521-1 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191997 | 4718 | - | Update to non-anchor carrier requirement for demod test cases. | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192050 | 4721 | - | Adding test case 6.5.2.1EC.1, Error Vector Magnitude (EVM) for UE category M2 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192071 | 4722 | - | Editorial Correction in TC 7.3A.3 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192073 | 4724 | - | Correction to dual layer spatial multiplexing TCs | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192090 | 4726 | - | Incorporating Test Tolerance into Test requirement for IM-DLCCH PCFICH-PDCCH Type A TCs | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192093 | 4727 | - | Update to test cases 6.2.2_1 and 6.2.3_1 in F.3.2 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192094 | 4728 | - | Incorporating Test Tolerance into Test requirement for IM-DLCCH PCFICH-PDCCH Type B TCs | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192096 | 4729 | - | Incorporating Test Tolerance into Test requirement for IM-DLCCH PHICH Type A TCs | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192098 | 4730 | - | Incorporating Test Tolerance into Test requirement for IM-DLCCH PHICH Type B TCs | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192100 | 4731 | - | Incorporating Test Tolerance into Test requirement for IM-DLCCH PEDPCCH Type A TCs | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192102 | 4732 | - | Test Tolerance and MU update for Interference Mitigation for Downlink Control Channels Demod TCs | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192111 | 4734 | - | Updating test case 6.2.4 Table 6.2.4.4.1-24 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192126 | 4736 | - | Editorial Correction in TC 7.3A.9 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192182 | 4738 | - | Addition of CQI Reporting under AWGN conditions with PUCCH 1-1 for FD-MIMO with channelMeasRestriction configured | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192446 | 4714 | 1 | Addition and correction of CA configuration for 3DL CA | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192447 | 4715 | 1 | Addition of CA configuration for 4DL CA | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192511 | 4672 | 1 | Updates of Rel-14 CA combinations | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192512 | 4659 | 1 | Update of TC 6.6.3G.2_2 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192513 | 4712 | 1 | Modification of Max Throughput averaging periodicity in A.8.2 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192514 | 4719 | 1 | Introduction of 4Rx TDD FDD CA TM9 demodulation test cases | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192515 | 4720 | 1 | Introduction of 4Rx TDD FDD CA TM9 demodulation test cases in Annex | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192517 | 4644 | 1 | Clarification of usage of narrowband index in feMTC test cases | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192518 | 4647 | 1 | Addition of new feMTC test case - OBW for category M2 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192519 | 4648 | 1 | Addition of new feMTC test case - SEM for category M2 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192520 | 4649 | 1 | Addition of new feMTC test case - ACLR for category M2 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192521 | 4650 | 1 | Addition of new feMTC test case - transmitter spurious emissions for category M2 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192522 | 4651 | 1 | Addition of new feMTC test case - spurious co-existence for category M2 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192523 | 4652 | 1 | Addition of new feMTC test case - transmit intermodulation for category M2 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192524 | 4671 | 1 | Adding test case 6.5.1EC, Frequency Error for UE category M2 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192532 | 4737 | 1 | Introduction of FD-MIMO test cases: Reporting of CSI-RS Resource Indicator (CRI) | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192533 | 4670 | 1 | Shared Risk clarification in TS 36.521-1 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192536 | 4725 | 1 | Introduction to band 73 to Blocking characteristics in TS36.521-1 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192540 | 4723 | 1 | Correction to NB-IoT Minimum Output Power TC | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192552 | 4716 | 1 | Update to CBSR configuration for 4 TX test cases | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192595 | 4733 | 1 | Updating test case 6.2.3A.2 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192848 | 4667 | 2 | Update the applicability of RF test cases related to FGI bits 103 and 104 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192633 | 4668 | 1 | Correction to CA_2A-5B-66A-66A configuration in TC7.3A.10 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192643 | 4696 | 1 | Clean up for Inter-band UL CA Tx Test Cases | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192644 | 4700 | 1 | Correction to 6.6.2.2A.2 Additional Spectrum Emission Mask for CA | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192664 | 4707 | 1 | Introduction of Maximum Power Reduction (MPR) for CA (3UL CA) | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191064 | 4642 | - | Aligning CA delta TiB in sub-clause 6.2.5.3 with TS 36.101 v16.0.0 | 15.5.0 | 16.0.0 |
| 2019-03 | RAN#83 | R5-191065 | 4643 | - | Aligning CA delta RiB in sub-clause 7.3.3 with TS 36.101 v16.0.0 | 15.5.0 | 16.0.0 |
| 2019-03 | RAN#83 | R5-191159 | 4656 | - | Align CA information in clause 5 with TS36.101 v16.0.0 | 15.5.0 | 16.0.0 |
| 2019-03 | RAN#83 | R5-191160 | 4657 | - | Align clause 7.3A.0 with TS36.101 v16.0.0 | 15.5.0 | 16.0.0 |
| 2019-03 | RAN#83 | R5-192634 | 4710 | 1 | CA_18A-42A - Updates of 7.3A.3 REFSENS | 15.5.0 | 16.0.0 |
| 2019-03 | RAN#83 | R5-192635 | 4713 | 1 | CA_3A-18A-42A - Updates of 7.3A.5 REFSENS | 15.5.0 | 16.0.0 |
| 2019-03 | RAN#83 | R5-192639 | 4669 | 1 | Band 53 introduction in TS 36.521-1 | 15.5.0 | 16.0.0 |
| 2019-06 | RAN#84 | R5-193592 | 4739 | - | Correction of the reference table number of uplink configuration | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-193604 | 4740 | - | Introduction of LTE Band 85 in clause 5 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-193605 | 4741 | - | Introduction of Band 85 for UE category M1 and M2 in clause 5.2E | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-193606 | 4742 | - | Introduction of Band 85 for UE category NB1 and NB2 in clause 5.2F | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-193673 | 4745 | - | Introduction of Band 85 for UE category NB1 and NB2 in transmit power test cases in clause 6.2 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-193674 | 4746 | - | Introduction of LTE Band 85 in spurious emission test cases in clause 6.6 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-193675 | 4747 | - | Introduction of LTE Band 85 in reference sensitivity test case 7.3 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-193679 | 4750 | - | Introduction of LTE Band 85 in In-band blocking and Out-of-band blocking test cases 7.6.1 and 7.6.2 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-193711 | 4751 | - | CA_1A-3A-41C - Updates of 7.3A.9 REFSENS | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-193950 | 4759 | - | Updating general demodulation clause for category M2 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-193952 | 4760 | - | Updates to 8.11.1.1.3.2, FDD and half-duplex FDD PDSCH Transmit Diversity 2x1 for UE category M2 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-193958 | 4762 | - | Updates to Annex G for category M2 test cases | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-193970 | 4763 | - | Addition of test requirements for reference sense and 2DL inter-band CA configurations | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-193971 | 4764 | - | Addition of test requirements for reference sense and 3DL intra-band contiguous CA configurations | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-193973 | 4765 | - | Addition of test configurations and test requirements for reference sense and 3DL intra-band contiguous CA and inter-band configurations | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-193974 | 4766 | - | Addition of test requirements for reference sense and 4DL intra-band contiguous CA configurations | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-193975 | 4767 | - | Addition of test configurations and test requirements for reference sense and 4DL intra-band contiguous CA and inter-band configurations | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-193976 | 4768 | - | Addition of test configurations and test requirements for reference sense and 5DL intra-band contiguous CA and inter-band configurations | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194022 | 4769 | - | CA_1A-3A-41A-42A - Updates of 7.3A.9 REFSENS | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194068 | 4771 | - | Updates of Rel-16 CA combination | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194071 | 4773 | - | Spurious update for band 53 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194178 | 4786 | - | Update of V2X RX Test Cases | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194206 | 4789 | - | Correction to test configuration tables for intra-contiguous 2CA Tx tests | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194208 | 4791 | - | Editorial correction to Table 7.3A.3.5-2 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194211 | 4794 | - | Correction to TDD FDD CA 4Rx Demodulation Tests | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194212 | 4795 | - | Correction to 4Rx CA demod test cases | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194213 | 4796 | - | Correction to LAA demodulation test cases | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194216 | 4799 | - | Clean up to default test settings of CA_XC-YA/XA-YC | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194217 | 4800 | - | Correction to TM4 2DL CA PDSCH demodulation tests | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194218 | 4801 | - | Correction to LAA CQI tests | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194236 | 4804 | - | Addition of 6-DL CA SDR test cases | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194332 | 4810 | - | Completion of category M1 CEModeB TCs | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194338 | 4811 | - | Resolve TC ID conflict in A-SEM test | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194339 | 4812 | - | Correction to subclause used in section 6 test cases | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194341 | 4813 | - | Addition of new TC 8.7.2.1_H.4 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194343 | 4814 | - | Addition of new TC 8.7.2.1_H.5 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194391 | 4816 | - | Update to the test requirements in the V2X test cases | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194406 | 4817 | - | Correction to NS_04 test points | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194408 | 4818 | - | Editorial correction to table A.3.9.1-2 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194422 | 4819 | - | Updates of 7.3A.10 REFSENS for R16 CA | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194431 | 4820 | - | Adding test case 6.5.2.1EC.2, PUSCH-EVM with exclusion period for UE category M2 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194525 | 4826 | - | Updating test case 6.2.4A.2 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194527 | 4827 | - | Updating test case 6.2.4A.2_1 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194531 | 4828 | - | Updating test case 6.2.4A.2_2 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194534 | 4830 | - | Updating test case 6.5.2A.1.1_2 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194540 | 4833 | - | Updating test case 6.5.2A.3.4 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194600 | 4852 | - | Updating test case 8.7.2.1 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194630 | 4855 | - | Correction of Test Case 6.3.4EB.2.1 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194738 | 4857 | - | Update to TC 6.7A.4 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194742 | 4861 | - | Update to TC 6.6.1A.4 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194745 | 4862 | - | Update to TC 6.6.3.2A.4 Spurious emission band UE co-existence for CA (3UL CA) | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194747 | 4864 | - | Introduction of FDD FD-MIMO Class B PMI test case | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194748 | 4865 | - | Introduction of FDD FD-MIMO 16Tx Class A multiple PMI test case | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194749 | 4866 | - | Update to FDD FD-MIMO 12Tx Class A PMI test case | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194750 | 4867 | - | Introduction of FD-MIMO test cases into Annex | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194900 | 4839 | 1 | Reference sensitivity level for CA -- inter-band DL CA without UL CA | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194901 | 4848 | 1 | Maximum Test System Uncertainty for receiver tests | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194928 | 4743 | 1 | Introduction of LTE Band 85 in transmit power test cases in clause 6.2 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194929 | 4744 | 1 | Introduction of Band 85 for UE category M1 and M2 in transmit power test cases in clause 6.2 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194930 | 4748 | 1 | Introduction of Band 85 for UE category M1 and M2 in reference sensitivity test cases 7.3EA and 7.3EC | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194964 | 4783 | 1 | New CA band combination CA_3A_7A_28A, CA_3X_7Y and CA_7X_28Y | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194965 | 4784 | 1 | New CA band combination CA_7A_22A; Updates of 7.3A Refsens | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195005 | 4785 | 1 | Addition of new CA band 2A-5A-29A for CA 3DL CA REFSENS | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195008 | 4854 | 1 | Correction of UL 64QAM Test Cases | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195022 | 4802 | 1 | Introduction of ON/OFF time mask for 4UL CA test case | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195058 | 4782 | 1 | Introduction of New TC7.3A.11 Reference sensitivity level for 6DL CA | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195059 | 4790 | 1 | Editorial correction to Table 7.3.3-1B | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195060 | 4761 | 1 | Adding 8.11.1.1.3.2_1, FDD and half-duplex FDD PDSCH Transmit Diversity 2x1 for UE category M2 CEmodeB | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195061 | 4774 | 1 | Addition of new feMTC test case - A-MPR for category M2 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195062 | 4775 | 1 | Addition of new feMTC test case - A-SEM for category M2 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195063 | 4776 | 1 | Addition of new feMTC test case - additional spurious for category M2 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195064 | 4777 | 1 | Addition of new UL feMTC test cases - Annex F | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195065 | 4778 | 1 | Correction in configuration table of MPR test for cat M2 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195066 | 4779 | 1 | Correction in configuration table of SEM and ACLR tests for cat M2 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195070 | 4825 | 1 | Correcting an equation in many Tx Requirement Test Cases | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195071 | 4834 | 1 | Updating test case 8.2.1.1.1_A.5 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195072 | 4835 | 1 | Updating test case 8.2.1.3.1_A.4 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195073 | 4849 | 1 | Updating test case 8.2.1.4.2_A.4 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195074 | 4850 | 1 | Updating test case 8.7.1.1_A.5 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195075 | 4863 | 1 | New TC 6.6.3.3A.4 Additional spurious emissions for CA (3UL CA) | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195076 | 4853 | 1 | Correction to test case 6.2.2A.4 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195083 | 4772 | 1 | Introduction of Maximum input level for 6DL CA | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195093 | 4758 | 1 | Adding band 4, 14, 71 to Rx/Tx test cases for cat NB2 and cat M2 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195100 | 4787 | 1 | Introduction of FDD PDSCH Single Antenna Port Performance for CA (6 DL CA) | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195101 | 4788 | 1 | Introduction of FDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (6DL CA) | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195102 | 4752 | 1 | Update of TC 6.3.5A.1.4 Power Control Absolute power tolerance for CA 3UL CA | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195103 | 4753 | 1 | Update of TC 6.3.5A.2.4 Power Control Relative power tolerance for CA 3UL CA | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195104 | 4754 | 1 | Update of TC 6.3.5A.3.4 Aggregate Power Control Tolerance for CA 3UL CA | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195105 | 4755 | 1 | Update of TC 6.6.2.3A.4 Adjacent Channel Leakage power Ratio for CA 3UL CA | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195106 | 4756 | 1 | Update of TC 6.6.3.1A.4 Transmitter Spurious emissions for CA 3UL CA | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195107 | 4858 | 1 | Update to TC 6.2.5A.5 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195108 | 4859 | 1 | Update to TC 6.6.2.1A.4 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195109 | 4860 | 1 | Update to TC 6.6.2.2A.4 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195111 | 4832 | 1 | Updating test case 6.5.2A.3.2 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195124 | 4868 | 1 | Update 4Rx Test Requirement for Band 30 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195125 | 4792 | 1 | Correction to message contents of 6.3.2F | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195126 | 4793 | 1 | Editorial correction to NB-IoT NPDCCH demodulation test cases | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195127 | 4798 | 1 | Core alignment of LAA demodulation test 8.4.3.1.1 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195197 | 4815 | 1 | Correction to MU for LTE UL CA | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195199 | 4757 | 1 | Editorial change replacing TBD in test case 6.2.5F, Configured UE transmitted Output Power for UE category NB1 and NB2 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195447 | 4797 | 1 | Correction to LAA demodulation test 8.3.3.1.2 | 16.0.0 | 16.1.0 |
| 2019-09 | RAN#85 | R5-195704 | 4896 | - | Addition of new TC 6.2.2A.5 of TS 36.521-1 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-195706 | 4898 | - | Addition of new TC 6.2.4A.5 of TS 36.521-1 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-195707 | 4899 | - | Addition of new TC 6.2.5A.5 of TS 36.521-1 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-195708 | 4900 | - | Addition of new TC 6.3.2A.5 of TS 36.521-1 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-195709 | 4901 | - | Addition of new TC 6.3.3A.5 of TS 36.521-1 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-195710 | 4902 | - | Update table 6.2.2A.0-1 of TS 36.521-1 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-195713 | 4903 | - | Update of V2X MOP test cases | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-195714 | 4904 | - | Update of V2X MPR test cases | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-195715 | 4905 | - | Update of V2X AMPR test cases | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-195717 | 4906 | - | Cleaning up of V2X test cases for missing applicability | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-195718 | 4907 | - | Addition of V2X Carrier leakage test cases | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-195719 | 4908 | - | Addition of V2X ACLR for multi-carrier | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-195720 | 4909 | - | Update of V2X OOBB test cases | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-195721 | 4910 | - | Update of V2X Intermodulation test cases | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-195722 | 4911 | - | Addition of MU and TT information of V2X test cases in Annex F | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-195724 | 4912 | - | Addition of V2X demod case 14.5 and 14.6 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-195725 | 4913 | - | Addition of V2X demod case 14.7 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-195795 | 4944 | - | Addition of inter_band CA configurations to the OOB blocking test which have restricted operating frequency range for Band 28 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-195802 | 4946 | - | Addition of CA configuration CA_11A_41C and CA_11A_42C for Refsens test | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-195840 | 4947 | - | CA_1A-18A-42A - Updates of 7.3A.5 REFSENS | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-195846 | 4948 | - | CA_18A-42C - Updates of 7.3A.5 REFSENS | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-195852 | 4950 | - | Correction of Test Setting CA_1A-3A-41C | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196061 | 4951 | - | Introduction of Aggregate Power Control Tolerance for 4UL CA test case | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196062 | 4952 | - | Introduction of power control absolute power tolerance for 4UL CA test case | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196063 | 4953 | - | Introduction of transmit intermodulation for 4UL CA test case | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196080 | 4954 | - | Aligning CA delta TiB in clause 6.2.5.3 with TS 36.101 v16.2.0 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196081 | 4955 | - | Aligning CA delta RiB in clause 7.3.3 with TS 36.101 v16.2.0 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196083 | 4956 | - | Introduction of uncertainties and TTs in Annex F for some 4UL CA test cases | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196142 | 4957 | - | New Tx test case 6.5.1A.5 Frequency Error for 4UL CA | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196143 | 4958 | - | New Tx test case 6.5.2A.1.5 Error Vector Magnitude (EVM) for 4UL CA | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196144 | 4959 | - | New Tx test case 6.5.2A.2.5 Carrier leakage for 4UL CA | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196145 | 4960 | - | New Tx test case 6.5.2A.3.5 In-band emissions for non allocated RB for 4UL CA | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196146 | 4961 | - | New Tx test case 6.6.1A.5 Occupied bandwidth for 4UL CA | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196147 | 4962 | - | Addition of Annex F of Tx test cases for 4UL CA | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196151 | 4963 | - | Correction to transmit modulation test cases for 3UL CA | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196222 | 4966 | - | Correction on NPRACH CP length configuration | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196224 | 4968 | - | Editorial correction to reference sensitivity for 4CA | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196225 | 4969 | - | Correction to protected band configuration for band 85 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196226 | 4970 | - | Editorial correction to G.3A.5 minimum test time | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196227 | 4971 | - | Clarifying test applicability selection criteria in FDD SDR TC 8.7.1.1 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196450 | 4974 | - | Introduction of FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for 6DL CA | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196506 | 4976 | - | Align CA information in clause 5 with core specificaiton | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196507 | 4977 | - | Align clause 7.3A.0 with core specificaiton | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196513 | 4978 | - | Introduction of TDD FDD CA PDSCH Single Antenna Port Performance for FDD PCell 6DL CA | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196515 | 4979 | - | Introduction of TDD FDD CA PDSCH Single Antenna Port Performance for TDD PCell 6DL CA | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196548 | 4980 | - | Update REFSENS of CA_8A-40C configuration in 7.3A.5 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196549 | 4981 | - | Update REFSENS of CA_8A-20A-28A configuration in 7.3A.5 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196550 | 4982 | - | Update REFSENS of CA_3A-28A-38A configuration in 7.3A.5 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196551 | 4983 | - | Update REFSENS of CA_1A-8A-38A configuration in 7.3A.5 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196552 | 4984 | - | Update REFSENS of CA_1A-1A configuration in 7.3A.4 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196553 | 4985 | - | Update REFSENS of CA_1A-1A-7A configuration in 7.3A.5 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196554 | 4986 | - | Update REFSENS of CA_3A-7A-38A configuration in 7.3A.5 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196555 | 4987 | - | Update REFSENS of CA_3A-3A-8A configuration in 7.3A.5 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196557 | 4989 | - | Update REFSENS of 2DL CA for multiple CA configurations in 7.3A.3 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196619 | 4993 | - | Updates of Rel-16 CA configuration | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196758 | 4997 | - | Addition of 8.2.2.8.1 and 8.2.2.8.2 into Annex F and Annex G | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196858 | 4998 | - | Correction to RMC for Cat M1 CSI tests | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196861 | 4999 | - | Updating test case 6.2.4A.2_2 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196863 | 5000 | - | Updating test case 6.6.1A.4 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197268 | 4917 | 1 | Adding CA_66B and CA_66C on Table 6.2.2A.1.5-1 CA UE Power Class test requirements | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197269 | 4919 | 1 | Adding test points on on Table 6.3.5A.3.1.4.1-2 Correction on Table 6.3.5A.3.1.4.1-2 changing the point 2575 into 75 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197310 | 5004 | - | Updating test cases to remove transient period for FGI 103 and 104 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197421 | 4972 | 1 | Update editors note for several 3UL CA RF conformance test cases | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197443 | 4943 | 1 | Addition of CA configuration CA_20A_40A for Refsens test | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197444 | 4988 | 1 | Update REFSENS of CA_28A-40D configuration in 7.3A.9 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197445 | 4996 | 1 | Addition of test cases 8.2.2.8.1 and 8.2.2.8.2 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197446 | 4964 | 1 | Editorial change, correcting reference in 7.3F.1.5 test requirement | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197447 | 4991 | 1 | Editorial change - moving test case 7.6.2AE to correct place | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197448 | 4975 | 1 | Update to FDD 6 DL CA Single Antenna and Open Loop Spatial Multiplexing 2x2 Performance test cases | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197449 | 4897 | 1 | Addition of new TC 6.2.3A.5 of TS 36.521-1 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197454 | 4939 | 1 | Adding Bandwidth combination for FDD CA on Table 8.1.2.2-2, Table 8.1.2.3-1 and Table 9.1.1.2-1 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197455 | 4940 | 1 | Adding Bandwidth combination for FDD CA on Table 8.1.2.2-2 for 7A-7A BW SET 3 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197458 | 4945 | 1 | Addition of CA configuration CA_11A_41A and CA_11A_42A for Refsens test | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197460 | 4914 | 1 | Addition of V2X demod case 14.9 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197461 | 4973 | 1 | Update for Message contents for V2X Test case in non-concurrent mode | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197465 | 4949 | 1 | CA_1A-3A-18A-42A - Updates of 7.3A.9 REFSENS | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197466 | 4990 | 1 | Correction of table 7.3A.0-3 in 36.521-1 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197467 | 4992 | 1 | Adding feMTC test case 7.6.2EC, Out-of-band blocking for UE category M2 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197470 | 4994 | 1 | Addition of FD-MIMO test cases 9.2.4.1A & 9.2.4.2A | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197489 | 5003 | 1 | Adding test points for CA_25A-41D to Table 7.3A.9.4.1-1: Test Configuration Table | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197605 | 4965 | 1 | Correction to test configuration tables for intra-contiguous 2CA Tx tests | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197606 | 4967 | 1 | Correction to test configuration for 4CA reference sensitivity | 16.1.0 | 16.2.0 |
| 2019-10 | RAN#85 | R5-196858 | 4998 | - | Fix wrong implementation of RC.25 FDD/TDD in Table A.4-1 | 16.2.0 | 16.2.1 |
| 2019-12 | RAN#86 | R5-197765 | 5006 | - | Cleaning up of R13 CA test cases | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-197875 | 5017 | - | Editorial change to test case 6.2.2EC | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-197898 | 5018 | - | Adding test case 8.11.1.1.3.2_1 to Annex F | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-197899 | 5019 | - | Updating test case 8.11.1.2.1 and 8.11.1.2.2 to be applicable for category M2 | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-197958 | 5020 | - | Alignment with core specification, adding missing clause 8.1.2.8A | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-197959 | 5021 | - | Updating test case 7.3EC, Reference sensitivity level for UE category M2 | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198074 | 5023 | - | Correction to 4Rx CA demod test case 8.13.1.3.1 and 8.13.1.4.1 | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198105 | 5024 | - | Correction of test configuration table for TC 7.6.3A.5 narrow band blocking for 3DL CA | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198109 | 5025 | - | Update to TDD FDD CA Single Antenna Port Performance test cases for 6 DL CA | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198110 | 5026 | - | Update to FDD 6 DL CA Closed Loop Spatial Multiplexing 4x2 Performance | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198111 | 5027 | - | Introduction of TDD FDD 6 DL CA PDSCH Open Loop Spatial Multiplexing 2x2 for TDD PCell Performance test case | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198112 | 5028 | - | Introduction of TDD FDD 6 DL CA PDSCH Open Loop Spatial Multiplexing 2x2 for FDD PCell Performance test case | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198292 | 5035 | - | Addition of new TC 8.7.2.1_H.6 | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198297 | 5036 | - | Addition of new TC 8.7.2.1_A.6 | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198345 | 5040 | - | Addition of new Rel-15 CA test case 7.5A.9 | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198347 | 5041 | - | Addition of new Rel-15 CA test case 7.8.1A.9 | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198357 | 5042 | - | Addition of out of band blocking for CA_48C | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198396 | 5043 | - | Correction to reference sensitivity for CA test cases | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198491 | 5047 | - | Correction to Spurious emission | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198515 | 5048 | - | Update to CBSR configuration for 4 TX test cases with 3 layers | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198672 | 5050 | - | New TDD FD-MIMO test cases in Annex | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198727 | 5052 | - | Editorial correction to reference sensitivity test case | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198728 | 5053 | - | Update to reference sensitivity test case | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199080 | 5039 | 2 | Addition of CA configuration CA_2A_4A_4A_5A for Refsens test | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199438 | 5007 | 1 | Addition of new feMTC test case - PRACH time mask | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199439 | 5008 | 1 | Addition of new feMTC test case - SRS time mask | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199440 | 5009 | 1 | Addition of new feMTC test case - carrier leakage | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199441 | 5010 | 1 | Addition of new feMTC test case - in-band emissions for non allocated RB | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199442 | 5011 | 1 | Addition of new feMTC test case - EVM equalizer spectrum flatness | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199443 | 5012 | 1 | Addition of new feMTC test case - frequency error for CE Mode B | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199444 | 5013 | 1 | Addition of new feMTC test case - aggregate power control tolerance for CE Mode B | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199445 | 5014 | 1 | Addition of new feMTC test case - spurious response | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199446 | 5015 | 1 | Addition of new feMTC test case - spurious emissions | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199447 | 5016 | 1 | Addition of new feMTC test cases - annex F | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199455 | 5029 | 1 | Update of SDR joint CA test cases | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199458 | 5037 | 1 | Addition of CA configuration CA_42A_42C for Refsens test | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199459 | 5038 | 1 | Addition of CA configuration CA_38A_40A_40A for Refsens test | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199463 | 5030 | 1 | CA_1A-18A-42C - Updates of 7.3A.9 REFSENS | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199464 | 5031 | 1 | CA_3A-18A-42C - Updates of 7.3A.9 REFSENS | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199465 | 5032 | 1 | CA_1A-3A-18A-42C - Updates of 7.3A.10 REFSENS | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199470 | 5022 | 1 | Adding new Bandwidth Combination for 3+5Mhz for CA_5B set 1 | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199471 | 5044 | 1 | Update of TC 8.2.2.1.1_A.4 TDD PDSCH Single Antenna Port Performance for CA (5DL CA) | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199474 | 5005 | 1 | Update of AMPR and SEM for RAN4 alignment | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199480 | 5049 | 1 | Introduction of new TDD FD-MIMO test cases | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199528 | 5045 | 1 | Update of Test Case 6.2.3A.4 Maximum Power Reduction for 3UL CA | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199529 | 5046 | 1 | Update of Test Case 6.2.4A.4 Additional Maximum Power Reduction (A-MPR) for CA (3UL CA) | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199558 | 5051 | 1 | Correction to Rx test points for CA_1A-3A-42C | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199559 | 5054 | 1 | Update to SEM and additional spurious emissions LTE CA 41C | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199560 | 5055 | 1 | CA_41X-41Y, in-band blocking, updated Table 7.6.1A.7.4.1-2: Test Configuration Table | 16.2.1 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199582 | 5033 | 1 | Update of 7.3A.3 Reference sensitivity level for CA Band 5A-29A | 16.2.1 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200291 | 5057 |  | Addition of new test case 8.11.1.2.3.2, TDD PDSCH 2 Tx Antenna Port supporting wideband transmission for UE category M2 | 16.3.0 | 16.4.0 |
| 2020-03 | RAN#87 | R5-200292 | 5058 |  | Addition of new test case 8.11.1.2.3.2_1, TDD PDSCH 2 Tx Antenna Port supporting wideband transmission for UE category M2 (CEModeB) | 16.3.0 | 16.4.0 |
| 2020-03 | RAN#87 | R5-200798 | 5062 |  | Update of FD-MIMO PMI test cases | 16.3.0 | 16.4.0 |
| 2020-03 | RAN#87 | R5-200890 | 5059 | 1 | Update to CRI reporting cases for FD-MIMO | 16.3.0 | 16.4.0 |
| 2020-03 | RAN#87 | R5-200891 | 5060 | 1 | Update to CQI Reporting cases with single CSI Process | 16.3.0 | 16.4.0 |
| 2020-03 | RAN#87 | R5-177392 | 3949 | 1 | Cat1bis: Addition of new CAT1-bis TxDiv Demod TC (was not implemented by mistake) | 16.3.0 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201694 | 5069 | - | Update REFSENS of some CA combinations in 7.3A.5 | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-201695 | 5070 | - | Update REFSENS of CA_38A-40C BCS1 configuration in 7.3A.5 | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-201700 | 5071 | - | Update of 6.2.2_1 MOP for HPUE | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-201701 | 5072 | - | Update of 6.2.3_1 MPR for HPUE | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-201730 | 5077 | - | Update of Tolerance for 8.2.1.10 in Annex F | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-201788 | 5083 | - | Updated to clause 5 information for Rel-15 NB-IoT | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-201803 | 5084 | - | Introduction of TDD FDD 6 DL CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for FDD PCell | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-201804 | 5085 | - | Introduction of TDD FDD 6 DL CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for TDD PCell | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-201805 | 5086 | - | Introduction of TDD FDD 7 DL CA PDSCH Open Loop Spatial Multiplexing 2x2 for FDD PCell | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-201806 | 5087 | - | Introduction of TDD FDD 7 DL CA PDSCH Open Loop Spatial Multiplexing 2x2 for TDD PCell | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-201807 | 5088 | - | Introduction of TDD FDD 7 DL CA PDSCH Single Antenna Port Performance for FDD PCell | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-201808 | 5089 | - | Introduction of TDD FDD 7 DL CA PDSCH Single Antenna Port Performance for TDD PCell | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-201809 | 5090 | - | Update to TDD FDD 6 DL CA PDSCH Open Loop Spatial Multiplexing 2x2 for FDD PCell | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-201810 | 5091 | - | Update to TDD FDD 6 DL CA PDSCH Open Loop Spatial Multiplexing 2x2 for TDD PCell | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-201831 | 5092 | - | Update of inband blocking requirements | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-201868 | 5094 | - | Moving test case 7.6.1EA In-band blocking for UE category M1 to correct place | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-201895 | 5101 | - | Addition of calculation of averaged EVM for short TTI | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-201928 | 5115 | - | Cleanup in category M2 test cases | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-201950 | 5116 | - | New RMCs for efeMTC | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202047 | 5117 | - | Skipping 2Rx REFSENS testing on bands where UE support 4Rx | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202174 | 5119 | - | Addition of new TC 8.7.1.1_A.7 | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202175 | 5120 | - | Addition of new TC 8.7.1.1_H.7 | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202176 | 5121 | - | Addition of new TC 8.7.2.1_A.7 | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202177 | 5122 | - | Addition of new TC 8.7.2.1_H.7 | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202198 | 5126 | - | Addition of RMC for TDD NB-IOT testing | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202280 | 5127 | - | Cleaning up DL CA Rx Test Cases | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202281 | 5128 | - | Correction to reference sensitivity CA_38A-40A-40A | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202282 | 5129 | - | Editorial correction to spurious emission UE-coexistence tests for Cat-M1 | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202283 | 5130 | - | Editorial correction to spurious emission UE-coexistence tests for UL CA | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202324 | 5131 | - | efeMTC updates in test case 8.11.1.1.3.1 | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202442 | 5136 | - | Corrections to Reference Sensitivity for CA | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202445 | 5137 | - | Editorial corrections to Reference Sensitivity for CA | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202705 | 5063 | 1 | Addition of CA_48C and CA_48D to 36.521-1 Table 7.6.2A.5.5-2 | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202758 | 5078 | 1 | correction of references in tc 6.2.3A.1_3 Maximum Power Reduction (MPR) for CA (intra-band contiguous DL CA and UL CA) for HPUE | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202759 | 5079 | 1 | Addition of new test case 6.2.4A.1_3 Additional Maximum Power Reduction (A-MPR) for CA (intra-band contiguous DL CA and UL CA) for HPUE | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202760 | 5080 | 1 | Addition of new test case 6.2.5A.1_3 Configured UE transmitted Output Power for CA (intra-band contiguous DL CA and UL CA) for HPUE | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202761 | 5081 | 1 | Addition of new test case 6.6.2.3A.1_3 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) for HPUE | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202767 | 5098 | 1 | Addition of 6.2.5_s Configured UE Transmitted Output Power for short TTI | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202768 | 5099 | 1 | Addition of 6.3.4.1_s General ON/OFF time mask for short TTI | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202770 | 5102 | 1 | Addition of 8.14.1.1.1 FDD sPDSCH Open Loop Spatial Multiplexing Performance | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202771 | 5103 | 1 | Addition of 8.14.1.1.2 FDD sPDSCH Closed Loop Spatial Multiplexing Performance | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202772 | 5104 | 1 | Addition of 8.14.1.2.1 TDD sPDSCH Open Loop Spatial Multiplexing Performance | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202773 | 5105 | 1 | Addition of 8.14.1.2.2 TDD sPDSCH Closed Loop Spatial Multiplexing Performance | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202774 | 5106 | 1 | Addition of 8.14.2.1 FDD sPDCCH Transmit diversity performance | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202775 | 5107 | 1 | Addition of 8.14.2.2 TDD sPDCCH Transmit diversity performance | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202776 | 5108 | 1 | Addition of 9.12.1.1 FDD CQI Reporting under fading conditions based on CRS for short TTI | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202777 | 5109 | 1 | Addition of 9.12.1.2 TDD CQI Reporting under fading conditions based on CRS for short TTI | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202778 | 5110 | 1 | Addition of 9.12.2.1 FDD CQI Reporting under fading conditions based on CSI-RS for short TTI | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202779 | 5111 | 1 | Addition of 9.12.2.2 TDD CQI Reporting under fading conditions based on CSI-RS for short TTI | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202780 | 5112 | 1 | Addition of Reference Measurement Channels for short TTI | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202785 | 5068 | 1 | Update for each NB1 and NB2 test cases in Annex F | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202786 | 5125 | 1 | Cleaning up NB-IOT Tx test cases to allow testing on TDD bands | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202793 | 5082 | 1 | Correction to test applicability of NB-IoT tests | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202794 | 5093 | 1 | Update to NS_05 Requirements | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202795 | 5123 | 1 | Adding NB-IOT ACLR for PC6 | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202796 | 5124 | 1 | Updated to NB-IOT TC 6.2.3FA | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202855 | 5095 | 1 | Addition of 6.2.2_s UE Maximum Output Power for short TTI | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202856 | 5097 | 1 | Addition of 6.2.4_s UE Additional Maximum Power Reduction for short TTI | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202857 | 5100 | 1 | Addition of 6.5.2.1_s Error Vector Magnitude for short TTI | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202858 | 5113 | 1 | Update of Annex C.3.2 Downlink Physical Channels for short TTI | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202863 | 5064 | 1 | Adding test band for UE Maximum Output Power for category NB1 and NB2 | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202864 | 5065 | 1 | Adding test band for UE Maximum Output Power for category NB1 and NB2 Power Class 6 | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202865 | 5066 | 1 | Adding test band for Reference sensitivity level without repetitions for category NB1 and NB2 | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202866 | 5067 | 1 | Adding test band for Out-of-band blocking for category NB1 and NB2 | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202867 | 5074 | 1 | Adding test band for ON OFF time mask for category NB1 and NB2 | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202869 | 5075 | 1 | Correction to TC 9.2.1.7 and 9.2.1.8 for minimum requirements | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202870 | 5076 | 1 | Addition of performance TC 8.2.1.10 intra-band contiguous CA with minimum channel spacing | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202877 | 5118 | 2 | correction in 6.2.2A.1_3 MOP for CA and HPUE (intra-band contiguous DL CA and UL CA) | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202935 | 5114 | 1 | Addition of B40 into UE Category 1bis and Addition of Power Class 2 into B41 UE Category 1bis | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-203090 | 5096 | 1 | Addition of 6.2.3_s UE Maximum Power Reduction for short TTI | 16.4.0 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203228 | 5138 | - | Introduction of PC2 UE for cat M1/M2 in LTE Bands 31 and 72 | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-203335 | 5139 | - | Addition of B34 into UE Category 1bis and update B41 UE Category 1bis | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-203561 | 5141 | - | Introduction of new TC 7.6.1A.9 In-band blocking for 6DL CA | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-203562 | 5142 | - | Introduction of new TC 7.6.2A.9 Out-of-band blocking for 6DL CA | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-203563 | 5143 | - | Introduction of new TC 7.6.3A.9 Narrow band blocking for 6DL CA | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-203564 | 5144 | - | Introduction of new TC 7.7A.9 Spurious response for 6DL CA | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-203570 | 5145 | - | Update of Annex F.1.3 and 3.3 for 6DL blocking and spurious response TCs | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-203600 | 5146 | - | Introduction of TDD FDD 7 DL CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for FDD PCell | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-203601 | 5147 | - | Introduction of TDD FDD 7 DL CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for TDD PCell | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-203602 | 5148 | - | Update to TDD FDD 6 DL CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for FDD PCell | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-203604 | 5149 | - | Update to TDD FDD 6 DL CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for TDD PCell | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-203606 | 5150 | - | Update to TDD FDD 7 DL CA PDSCH Open Loop Spatial Multiplexing 2x2 for FDD PCell | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-203607 | 5151 | - | Update to TDD FDD 7 DL CA PDSCH Open Loop Spatial Multiplexing 2x2 for TDD PCell | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-203608 | 5152 | - | Update to TDD FDD 7 DL CA Single Antenna Port Performance for FDD PCell | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-203609 | 5153 | - | Update to TDD FDD 7 DL CA Single Antenna Port Performance for TDD PCell | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-203738 | 5154 | - | Cleanup in test case 7.3EC, Reference sensitivity level for UE category M2 | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-203775 | 5155 | - | Updating test case applicability for CSI test cases | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-203776 | 5156 | - | Adding test case applicability rules | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-203779 | 5157 | - | Adding tests for CRS muting, 64QAM and 200Hz doppler spread to test case 8.11.1.1.3.1 | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-203782 | 5158 | - | Adding tests for CRS muting, 64QAM and 200Hz doppler spread to test case 8.11.1.2.3.1 | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-203857 | 5163 | - | Update of Test Tolerance for HPUE | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-203858 | 5164 | - | Update of MOP for HPUE | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-203859 | 5165 | - | Update of MPR for HPUE | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-203927 | 5166 | - | Addition of sTTI related definitions, symbols and abbreviations to TS 36.521-1 | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-203928 | 5167 | - | Update of message contents and test tolerance for transmitted power in sTTI test cases | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-203930 | 5169 | - | Update of test tolerance for CSI reporting in sTTI test cases | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204001 | 5177 | - | Annex F based on updated Tx test cases for efeMTC | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204005 | 5179 | - | Adding PC6 to 6.3.5EA.1, Power Control Absolute power tolerance for UE category M1 | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204007 | 5180 | - | Adding PC6 to 6.2.5EA, Configured UE transmitted Power for UE category M1 | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204017 | 5184 | - | Adding PC6 to 6.3.5EA.2, Power Control Relative power tolerance for UE category M1 | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204157 | 5185 | - | Correction to RF NB-IoT test cases | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204275 | 5187 | - | Corrections to Reference Sensitivity for CA | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204750 | 5176 | 1 | Updating test cases for several CA configurations | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204830 | 5171 | 1 | Updated to NB-IOT TC 6.3.5F.1 | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204831 | 5186 | 1 | Updated to test applicability for R15 NB-IoT requirement | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204841 | 5173 | 1 | Cleaning up of V2X RB allocation for non-concurrent transmission | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204842 | 5174 | 1 | Cleaning up of V2X RB allocation for simultaneous transmission | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204843 | 5175 | 1 | Cleaning up of V2X RB allocation for multi-carrier operation | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204892 | 5168 | 1 | Update of test tolerance for demodulation performance in sTTI test cases | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204893 | 5170 | 1 | Addition of acceptable uncertainty and test tolerance for sTTI test cases | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204965 | 5172 | 1 | Updated to NB-IOT TC 6.3.5F.3 | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204967 | 5159 | 1 | Addition of test case 9.8.3.1 FDD and Half duplex FDD CQI reporting definition under AWGN conditions for UE supporting coverage enhancement and 64QAM | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204968 | 5160 | 1 | Addition of test case 9.8.3.2 TDD CQI reporting under AWGN conditions for UE supporting coverage enhancement and 64QAM | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204969 | 5161 | 1 | Addition of test case 9.8.4.1 FDD and half duplex FDD CQI reporting under AWGN conditions for UE supporting coverage enhancement and alternative table | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204970 | 5162 | 1 | Addition of test case 9.8.4.2TDD CQI reporting under AWGN conditions for UE supporting coverage enhancement and alternative table | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204971 | 5178 | 1 | Adding sub PRB allocation to 6.5.2.3EA, In-band emissions for non-allocated RB for UE category M1 | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204972 | 5182 | 1 | Adding PC6 to 6.2.2EA, UE Maximum Output Power for UE category M1 | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204973 | 5183 | 1 | Adding PC6 to 6.2.3EA, Maximum Power Reduction (MPR) for UE category M1 | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204983 | 5140 | 2 | Editorial correction to 3DL CA test cases | 16.5.0 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205079 | 5188 | - | Introduction of LTE Bands 87 and 88 in clause 5 | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205080 | 5189 | - | Introduction of Bands 87 and 88 for UE category M1/M2 and NB1/NB2 | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205081 | 5190 | - | Introduction of LTE Bands 87 and 88 in transmit power test cases in clause 6.2 | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205082 | 5191 | - | Introduction of LTE Bands 87 and 88 in spurious emission test cases in clause 6.6 | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205083 | 5192 | - | Introduction of LTE Bands 87 and 88 in reference sensitivity test case 7.3 | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205085 | 5194 | - | Introduction of LTE Bands 87 and 88 in In-band blocking and Out-of-band blocking test cases 7.6.1 and 7.6.2 | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205107 | 5195 | - | UL-CA_41C changes to additional spurious emissions for QPSK and 64 QAM | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205119 | 5196 | - | Addition of B48 to Table 7.6.1A.3.5-2 | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205120 | 5197 | - | Addition of B48 to Table 7.6.1A.8.5-5 | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205122 | 5198 | - | Addition of B48 to Note 5 in Table 7.6.2A.3.5-2 | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205131 | 5199 | - | Addition of B48 to Note 5 in Table 7.6.2A.5.5-4 and to Table 7.6.2A.5.5-5 | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205153 | 5200 | - | Addition of B41 and B48 to Table 7.6.2A.7.5-2 and B48 to Tables 7.6.2A.7.5-4 and 7.6.2A.7.5-5 | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205154 | 5201 | - | Addition of B48 to Tables 7.6.2A.8.5-4 and 7.6.2A.8.5-5 | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205272 | 5204 | - | Update to 6 DL CA test cases for the test applicability | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205274 | 5205 | - | Update to 7 DL CA Open Loop Spatial Multiplexing 2x2 for the test applicability | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205278 | 5206 | - | Update to 7 DL CA Single Antenna Port Performance for the test applicability | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205281 | 5207 | - | Update to TDD FDD 7 DL CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for FDD PCell | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205282 | 5208 | - | Update to TDD FDD 7 DL CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for TDD PCell | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205362 | 5209 | - | Update to TDD FDD 6 DL CA test cases for the test applicability | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205525 | 5210 | - | Update of TC8.2.2.3.1_A.3 TDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (4DL CA) | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205526 | 5211 | - | Update of TC8.2.2.3.1_A.4 TDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (5DL CA) | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205538 | 5212 | - | Adding PC6 to 6.2.2EC UE Maximum Output Power for UE category M2 | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205539 | 5213 | - | Adding PC6 to 6.2.3EC, Maximum Power Reduction (MPR) for UE category M2 | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205542 | 5214 | - | Adding PC6 to 6.2.5EC Configured UE transmitted Power for UE category M2 | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205543 | 5215 | - | New subtest for test case 8.11.1.1.3.2_1 FDD and half-duplex FDD PDSCH Transmit Diversity 2x1 for UE category M2 (CEmodeB) | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205545 | 5216 | - | New subtest for test case 8.11.1.2.3.1_1 TDD PDSCH Transmit Diversity for UE category M1 (CEModeB) | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205546 | 5217 | - | Incorrect test requirement for power class 5 in test case 6.2.5EA | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205548 | 5218 | - | Alignment with core specification of clause 9.8 in 36.521-1 | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205549 | 5219 | - | Editorial, wrong references in test case 6.3.5EC Power Control for UE category M2 | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205688 | 5220 | - | Update to test applicability of NB-IoT RF tests in clause 6 | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205689 | 5221 | - | Update to test applicability of NB-IoT RF tests in clause 7 | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205690 | 5222 | - | Update to test applicability of NB-IoT RF tests in clause 8 | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205705 | 5223 | - | Updates for A-MPR and A-SEM test cases | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205751 | 5224 | - | Editorial. Correcting the table number for A-MPR for CA_NS_04 CA_41C (6.2.4A.1_1.3.4) | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205764 | 5225 | - | Update of test applicability in RF test cases for sTTI | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205765 | 5226 | - | Update of Demod test cases for sTTI across clause 8 and clause 9 | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205847 | 5227 | - | Correction of frequency spacing for demodulation of NB-IoT TC 8.12.1.1.2 | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205848 | 5228 | - | Correction of UL allocations for In-band emissions TC 6.5.2A.3.1 | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205899 | 5229 | - | Update of V2X MOP and MPR test cases | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205901 | 5230 | - | Alignment of V2X A-MPR, A-SEM and A-SE | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205902 | 5231 | - | Cleaning up of V2X test configurations | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205903 | 5232 | - | Clarification of V2X spurious emission measurement method | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205977 | 5233 | - | Update of 6.2.2B_1 MOP for UL-MIMO | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205982 | 5238 | - | Update of 6.3.5_1 Power Control for HPUE | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205984 | 5240 | - | Update of Annex F for HPUE Test Tolerance | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-206211 | 5241 | - | Update of test configuration in TC 6.3.5A.2.2 | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-206731 | 5234 | 1 | Correction of 6.2.3_1 MPR | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-206732 | 5235 | 1 | Update of 6.2.4_1 A-MPR | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-206733 | 5236 | 1 | Update of 6.2.5_1 configured power | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-206734 | 5237 | 1 | Update of 6.2.5A.1_3 Configured UE transmitted Output Power for CA and HPUE | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-206735 | 5239 | 1 | Update of 6.6.3.3 additional spurious emission | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-206770 | 5193 | 1 | Introduction of Bands 87 and 88 for UE category M1 and M2 in reference sensitivity test cases 7.3EA and 7.3EC | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-206896 | 5202 | 1 | Adding new TC 9.6.1.1_A.5 6DL CA | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-206897 | 5203 | 1 | Adding new TC 9.6.1.2_A.5 6DL CA | 16.6.0 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210262 | 5244 | - | Add support for new CA combos to Table 7.4A.8.4.1-1 and Table 7.4A.8.4.2-1 | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-210404 | 5246 | - | Introduction of FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (7DL CA) | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-210405 | 5247 | - | Introduction of FDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (7DL CA) | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-210406 | 5248 | - | Introduction of FDD PDSCH Single Antenna Port Performance for CA (7DL CA) | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-210407 | 5249 | - | Update to Applicability and test rules for different CA configurations for 6DL CA test cases | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-210409 | 5250 | - | Update to TDD FDD 7DL CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 Performance test cases | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-210410 | 5251 | - | Update to TDD FDD 7DL CA PDSCH Open Loop Spatial Multiplexing 2x2 Performance test cases | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-210411 | 5252 | - | Update to TDD FDD 7DL CA Single Antenna Port Performance test cases | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-210549 | 5253 | - | Adding subPRB allocation to test case 6.2.3EC, Maximum Power Reduction (MPR) for UE category M2 | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-210550 | 5254 | - | Core spec alignment, adding missing note in test case 8.11.1.2.3.1 | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-210552 | 5255 | - | Adding subPRB allocation to 6.5.2.1EA.2, PUSCH-EVM with exclusion period for UE category M1 | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-210553 | 5256 | - | Adding subPRB allocation to 6.5.2.2EA and 6.5.2.2EC | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-210558 | 5257 | - | Adding subPRB allocation to 6.6.2.1EC, Spectrum Emission Mask for UE category M2 | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-210559 | 5258 | - | Adding subPRB allocation to 6.6.2.3EC, Adjacent Channel Leakage power Ratio for UE category M2 | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-210560 | 5259 | - | Adding subPRB allocation to 6.6.3EA.1, Transmitter Spurious emissions for UE category M1 | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-210562 | 5260 | - | Adding band 7,42, 43 to Rx test cases for cat M2 | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-210776 | 5261 | - | Correction to the physical configuration for sTTI RF test cases | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-210777 | 5262 | - | Correction to the physical configuration for sTTI Demod test cases | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-210778 | 5263 | - | Correction to the physical configuration for sTTI CQI reporting test cases | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-210875 | 5269 | - | Update of 6.6.2.3_1 ACLR for HPUE | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-211257 | 5274 | - | Update for Band 48 in test case 6.2.2 | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-211258 | 5275 | - | Editorial Correction in test case 7.3A9 | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-211749 | 5242 | 1 | Addition of new test case 9.6.1.1_A.6 | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-211750 | 5243 | 1 | Addition of new test case 9.6.1.2_A.6 | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-211751 | 5245 | 1 | Addition of the definition of CA capability with 6DL and 7DL CCs | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-211755 | 5264 | 1 | Addition of new test case 6.6.2.1_s SEM for sTTI | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-211756 | 5266 | 1 | Addition of new test case 6.6.2.3_s ACLR for sTTI | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-211783 | 5268 | 1 | Update of 6.2.3A.1_3 Maximum Power Reduction for CA and HPUE | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-211834 | 5271 | 1 | Addition of band 7, 42, 43 and 65 for UE category M1 and NB1 to clause 5 | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-211835 | 5272 | 1 | Addition of band 7, 42, 43 and 65 for UE category M1 and NB1 to clause 6 | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-211903 | 5265 | 1 | Addition of new test case 6.6.2.2_s A-SEM for sTTI | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-211920 | 5270 | 1 | Correction to A-MPR test requirement of NS_22 in TC 6.2.4 | 16.7.0 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212049 | 5276 | - | Addition of 41A-48A to Table 7.3A.3.5-1 | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212050 | 5277 | - | Addition of 41-48 combos to Table 7.3A.5.5-5 | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212052 | 5279 | - | Add support for 41-48 CA combos to Table 7.3A.10.5-4 | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212054 | 5280 | - | Editorial of section header in Table 7.3A.10.4.1-1 to indicate support for existing default combos | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212055 | 5281 | - | Addition of XC-YD and YD-XC default test settings to Table 7.5A.8.4.1-1 and new CA configurations to Table 7.5A.8.4.2-1 | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212056 | 5282 | - | Addition of XC-YD and YD-XC default test settings to Table 7.6.1A.8.4.1-1 and new CA configurations to Table 7.6.1A.8.4.2-1 | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212057 | 5283 | - | Add Band 48 to Note 5 in Table 7.6.2A.0-0 | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212058 | 5284 | - | Addition of XC-YD and YD-XC default test settings to Table 7.6.2A.8.4.1-1 and new CA configurations to Table 7.6.2A.8.4.2-1 | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212059 | 5285 | - | Addition of XC-YD and YD-XC default test settings to Table 7.6.3A.8.4.1-1 and to Table 7.6.3A.8.4.2-1 | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212061 | 5286 | - | Addition of XC-YD and YD-XC default test settings to Table 7.8.1A.8.4.1-1 and Table 7.8.1A.8.4.2-1 | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212402 | 5291 | - | Introduction of TDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (6DL CA) | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212403 | 5292 | - | Introduction of TDD PDSCH Single Antenna Port Performance for CA (6DL CA) | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212404 | 5293 | - | Introduction of TDD PDSCH Single Antenna Port Performance for CA (7DL CA) | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212405 | 5294 | - | Introduction of TDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (7DL CA) | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212406 | 5295 | - | Update to FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (7DL CA) | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212407 | 5296 | - | Update to FDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (7DL CA) | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212408 | 5297 | - | Update to FDD PDSCH Single Antenna Port Performance for CA (7DL CA) | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212602 | 5298 | - | Correction of band 71 in cat M2 MPR test case, 6.2.3EC | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212630 | 5299 | - | Updating annex F for cat NB2 test cases for new frequency bands | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212633 | 5301 | - | Adding PC6 in efeMTC Tx test cases | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212639 | 5303 | - | Adding subPRB allocation in efeMTC Tx test cases | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212641 | 5304 | - | Remove CRS muting from CE mode B test cases | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212672 | 5305 | - | Addition of band 42 and 43 to A-MPR tests for a UE category M1 | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212673 | 5306 | - | Addition of band 7, 42, 43 and 65 to spurious emission for UE category M1 | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212752 | 5311 | - | Addition of new TC 7.6.1A.10 In-band blocking for 7DL CA | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212753 | 5312 | - | Addition of new TC 7.6.2A.10 OOB blocking for 7DL CA | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212754 | 5313 | - | Addition of new TC 7.6.3A.10 Narrow band blocking for 7DL CA | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212755 | 5314 | - | Addition of new TC 7.7A.10 Spurious response for 7DL CA | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212757 | 5315 | - | Addition of Table 7.1-2e for 7DL CA configurations | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212758 | 5316 | - | Update to Annexes F1.3 and F3.3 for 7DL TCs 7.6.1A.10 7.6.2A.10 7.6.3A.10 and 7.7A.10 | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212967 | 5317 | - | Correction to 6.6.2.3A.1_3 ACLR for CA for HPUE | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212968 | 5318 | - | Update of 6.2.3A.1_3 Maximum Power Reduction for CA and HPUE | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-213317 | 5320 | - | Correction of RMCs for NB-IoT | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-213830 | 5289 | 1 | Update to TDD sustained data rate performance for CA (5DL CA) | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-213831 | 5290 | 1 | Update to TDD TDD CQI Reporting under AWGN conditions PUCCH 1-0 for CA (5DL CA) | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-213963 | 5278 | 1 | Addition of XC-YC default test settings to Table 7.3A.9.4.1-1 and 41-48 combos to Table 7.3A.9.5-5 | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-213991 | 5302 | 1 | Adding RMCs for subPRB allocation | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-214014 | 5300 | 1 | Updating annex F for cat M1 test cases for new frequency bands | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-214015 | 5307 | 1 | Addition of band 7, 42, 43, 65 to spurious emission band UE co-existence for cat NB1 and NB2 | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-214033 | 5287 | 1 | Cleanup for spurious emission for UE co-existence table for LTE non CA | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-214034 | 5288 | 1 | Cleanup for spurious emission for UE co-existence table for LTE CA bands | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-214035 | 5321 | 1 | Update of spurious emission band UE co-existence | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-214036 | 5319 | 1 | Correction of Reference Sensitivity for CA test cases | 16.8.0 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214329 | 5322 | - | Introduction of eFD-MIMO in FDD demod test with aperiodic ZP CSI-RS | 16.9.0 | 16.10.0 |
| 2021-09 | RAN#93 | R5-214330 | 5323 | - | Introduction of eFD-MIMO in TDD demod test with aperiodic ZP CSI-RS | 16.9.0 | 16.10.0 |
| 2021-09 | RAN#93 | R5-214714 | 5324 | - | Adding test point with subPRB allocation in test case 6.2.3EA, 6.6.2.1EA, 6.6.2.3EA | 16.9.0 | 16.10.0 |
| 2021-09 | RAN#93 | R5-214716 | 5325 | - | Added subPRB allocation to cat M1 test cases, 6.2.4EA, 6.6.2.2EA and 6.6.3EA.3 for NS_07 | 16.9.0 | 16.10.0 |
| 2021-09 | RAN#93 | R5-214717 | 5326 | - | Brackets for MPR values for subPRB allocation removed | 16.9.0 | 16.10.0 |
| 2021-09 | RAN#93 | R5-214718 | 5327 | - | Correcting test point for subPRB allocation in test case 6.5.2.1EA.2 | 16.9.0 | 16.10.0 |
| 2021-09 | RAN#93 | R5-214722 | 5328 | - | Updates to Tx test case 6.6.3EA.2 for release-16 | 16.9.0 | 16.10.0 |
| 2021-09 | RAN#93 | R5-214723 | 5329 | - | Updates to Rx test cases 7.3EA, 7.6.1EA, 7.6.2EA for new release-16 bands for cat M1 | 16.9.0 | 16.10.0 |
| 2021-09 | RAN#93 | R5-214844 | 5331 | - | Cleanup for TS 36.521-1 spurious emission for UE co-existence for CA tables | 16.9.0 | 16.10.0 |
| 2021-09 | RAN#93 | R5-215040 | 5333 | - | Addition of band 42 and 43 to A-MPR tests for a UE category M2 | 16.9.0 | 16.10.0 |
| 2021-09 | RAN#93 | R5-215106 | 5334 | - | Adding NB TDD testing into 7.6.1F | 16.9.0 | 16.10.0 |
| 2021-09 | RAN#93 | R5-215109 | 5335 | - | Cleaning up UE categories requirement for 16QAM test points | 16.9.0 | 16.10.0 |
| 2021-09 | RAN#93 | R5-215122 | 5337 | - | Editorial, correcting format in test case 6.6.3EA.3 | 16.9.0 | 16.10.0 |
| 2021-09 | RAN#93 | R5-215331 | 5345 | - | Correction to Reference Channel Parameters in UE Category M1 | 16.9.0 | 16.10.0 |
| 2021-09 | RAN#93 | R5-215361 | 5346 | - | Updates to Tx test case 6.6.3EC.2 for release-16 | 16.9.0 | 16.10.0 |
| 2021-09 | RAN#93 | R5-215374 | 5347 | - | Update to V2X test cases | 16.9.0 | 16.10.0 |
| 2021-09 | RAN#93 | R5-215482 | 5348 | - | Editorial corrections in Reference Sensitivity for CA tests | 16.9.0 | 16.10.0 |
| 2021-09 | RAN#93 | R5-215484 | 5350 | - | Correction for CA_21A-28A in test case 7.3A.3 | 16.9.0 | 16.10.0 |
| 2021-09 | RAN#93 | R5-215922 | 5332 | 1 | Update of Rx Test Cases for CA_66A-66A-66A | 16.9.0 | 16.10.0 |
| 2021-09 | RAN#93 | R5-216004 | 5330 | 1 | Cleanup for TS 36.521-1 spurious emission for UE co-existence table (non CA) | 16.9.0 | 16.10.0 |
| 2021-09 | RAN#93 | R5-216005 | 5349 | 1 | Update of MPR, ACLR, SEM CA test cases | 16.9.0 | 16.10.0 |
| 2021-09 | RAN#93 | R5-216026 | 5338 | 1 | Update of Test Case 6.2.4A.4 Additional Maximum Power Reduction (A-MPR) for CA (3UL CA) | 16.9.0 | 16.10.0 |
| 2021-09 | RAN#93 | R5-215993 | 5339 | 1 | Updates to LTE band 24 - common | 16.10.0 | 17.0.0 |
| 2021-09 | RAN#93 | R5-215994 | 5340 | 1 | Updates to MOP and MPR test cases for LTE band 24 | 16.10.0 | 17.0.0 |
| 2021-09 | RAN#93 | R5-215995 | 5341 | 1 | Updates to A-MPR test cases for LTE band 24 | 16.10.0 | 17.0.0 |
| 2021-09 | RAN#93 | R5-215996 | 5342 | 1 | Updates to Additional Spurious Emission test cases for LTE Band 24 | 16.10.0 | 17.0.0 |
| 2021-09 | RAN#93 | R5-215997 | 5343 | 1 | Updates to reference sensitivity test case for LTE band 24 | 16.10.0 | 17.0.0 |
| 2021-09 | RAN#93 | R5-215998 | 5344 | 1 | Updates to In-band blocking test case for LTE band 24 | 16.10.0 | 17.0.0 |
| 2021-12 | RAN#94 | R5-216427 | 5351 | - | Addition of A-MPR test configuration and test requirements for NS_56 (B24) | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-216429 | 5353 | - | Addition of A-MPR with PUSCH frequency hopping test requirements for NS_56 (B24) | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-216431 | 5355 | - | Addition of A-MPR for UL MIMO test configuration and test requirements for NS_56 (B24) | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-216432 | 5356 | - | Derivation of A-MPR Test Requirements for NS_56 (B24) | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-216433 | 5357 | - | Additional spurious emissions test configuration for NS_56 (B24) | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-216436 | 5360 | - | Additional spurious emissions for UL MIMO test configuration for NS_56 (B24) | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-216527 | 5361 | - | Updates to UL-MIMO MOP and MPR test cases for LTE band 24 | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-216528 | 5362 | - | Correction to reference sensitivity test case for LTE band 24 | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-216781 | 5363 | - | Correction for CA_66 coexistence | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-216782 | 5364 | - | Removal of BCS1 for CA_5B | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-217090 | 5369 | - | Correction of test requirement for CA_66A-66A-66A | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-217119 | 5371 | - | Test case 6.2.4EC and 6.6.3EC.3 updated with subPRB allocation | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-217120 | 5372 | - | Test case 6.2.4EA, 6.6.2.2EA and 6.6.3EA.3 updated with NS values for subPRB allocation | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-217191 | 5374 | - | Update applicability of TC 9.6.1.1_A.5 | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-217193 | 5376 | - | Update applicability of TC 9.6.1.2_A.5 | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-217194 | 5377 | - | Update applicability of TC 9.6.1.2_A.6 | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-217291 | 5378 | - | CR implementation correction to add test case 7.4A.9 and corrections to 7.4A_9_H for 6DL CA | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-217398 | 5379 | - | Removing R15 V2X requirements | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-217399 | 5380 | - | Correction to V2X OBW for intra-band multi-carrier | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-217400 | 5381 | - | Cleaning up description of TDD slots in V2X test cases | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-217426 | 5383 | - | Correction to test configuration table in 6.5.2A.2.2 | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-217462 | 5386 | - | Addition of new LTE_CA_R15 test case - A-SEM for 4UL CA | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-217464 | 5388 | - | Addition of new LTE_CA_R15 test case - transmitter spurious emissions for 4UL CA | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-217465 | 5389 | - | Addition of new LTE_CA_R15 test case - spurious co-existence for 4UL CA | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-217466 | 5390 | - | Addition of new LTE_CA_R15 test case - A-spurious for 4UL CA | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-217467 | 5391 | - | Addition of new LTE_CA_R15 test cases - Annex F | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-217627 | 5393 | - | Corrections of some clause numbers for UTRA test requirements | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-218263 | 5385 | 1 | Addition of new LTE_CA_R15 test case - SEM for 4UL CA | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-218264 | 5387 | 1 | Addition of new LTE_CA_R15 test case - ACLR for 4UL CA | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-218374 | 5352 | 1 | Addition of A-MPR for UL 64QAM test configuration and test requirements for NS_56 (B24) | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-218375 | 5354 | 1 | Addition of A-MPR for UL 256QAM test configuration and test requirements for NS_56 (B24) | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-218376 | 5358 | 1 | Additional spurious emissions for UL 64QAM test configuration for NS_56 (B24) | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-218377 | 5359 | 1 | Additional spurious emissions for UL 256QAM test configuration for NS_56 (B24) | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-218391 | 5370 | 1 | Editorial, correcting test case header for 6.2.4E | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-218392 | 5392 | 1 | Update of MPR, ACLR, SEM CA test cases | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-218393 | 5395 | 1 | NB-IoT Test Frequency Corrections to align with RAN4 and US FCC | 17.0.0 | 17.1.0 |
| 2021-12 | RAN#94 | R5-218482 | 5394 | 1 | Addition of UE co-existence requirements for band 40 to TS 36.521-1 | 17.0.0 | 17.1.0 |
| 2022-03 | RAN#95 | R5-220448 | 5397 | - | Correction of UL CA configurations for CA_25A-41A,CA_25A-25A-41A, CA_25A-41C, CA_25A-25A-41C and CA_25A-41D in Table 5.4.2A.1-2 | 17.1.0 | 17.2.0 |
| 2022-03 | RAN#95 | R5-220675 | 5399 | - | Addition of new test case in Annex F.1.5 and Annex F.3.5 | 17.1.0 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221144 | 5400 | - | Update to maximum output power for UL CA Band combo 2A-46A | 17.1.0 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221364 | 5402 | - | Correction of RB allocation in LTE MPR CA test case | 17.1.0 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221754 | 5398 | 1 | Removing editors note in CA test cases | 17.1.0 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221918 | 5396 | 1 | Correction of 6.6.2.2_2 Additional Spectrum Emission Mask | 17.1.0 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221919 | 5401 | 1 | Correction of maximum output power for CA test case | 17.1.0 | 17.2.0 |
| 2022-06 | RAN#96 | R5-222433 | 5403 | - | Correction to RB allocation for CEMode B TCs | 17.2.0 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222434 | 5404 | - | Addition of missing RMC R.86 TDD | 17.2.0 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222542 | 5405 | - | Editorial correction in Refsens CA test case | 17.2.0 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222693 | 5406 | - | Update to test applicability for 4G test cases without UL CA | 17.2.0 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222697 | 5407 | - | Editorial correction to NB-IoT performance test cases | 17.2.0 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222698 | 5408 | - | Correction to MPR test applicability for category 1bis | 17.2.0 | 17.3.0 |
| 2022-09 | RAN#97 | R5-224143 | 5411 | - | Introduction of Band 103 for UE category NB1 and NB2 in transmit power test cases in clause 6.2 | 17.3.0 | 17.4.0 |
| 2022-09 | RAN#97 | R5-224414 | 5413 | - | Addition of 7DL CA Sustained data rate performance test case 8.7.5.1.6 | 17.3.0 | 17.4.0 |
| 2022-09 | RAN#97 | R5-224415 | 5414 | - | Addition of 7DL CA Sustained data rate performance test case 8.7.5.2.6 | 17.3.0 | 17.4.0 |
| 2022-09 | RAN#97 | R5-224623 | 5416 | - | Correction to lower limit for NS_56 in 6.2.4 | 17.3.0 | 17.4.0 |
| 2022-09 | RAN#97 | R5-224624 | 5417 | - | Correction to RB allocation for NS_03 NS_07 in UL CA test cases | 17.3.0 | 17.4.0 |
| 2022-09 | RAN#97 | R5-225116 | 5419 | - | Correction of test case 6.6.3EB.3 | 17.3.0 | 17.4.0 |
| 2022-09 | RAN#97 | R5-225120 | 5420 | - | Update of CA name release mapping | 17.3.0 | 17.4.0 |
| 2022-09 | RAN#97 | R5-225210 | 5421 | - | Corrections to Reference Sensitivity for CA | 17.3.0 | 17.4.0 |
| 2022-09 | RAN#97 | R5-225699 | 5412 | 1 | Correction of clause number for 4UL CA test requirement | 17.3.0 | 17.4.0 |
| 2022-09 | RAN#97 | R5-225777 | 5410 | 1 | Introduction of LTE Band 103 in clause 5 | 17.3.0 | 17.4.0 |
| 2022-09 | RAN#97 | R5-225778 | 5415 | 1 | Introduction of LTE Band 103 in spurious emission test cases in clause 6.6 | 17.3.0 | 17.4.0 |
| 2022-09 | RAN#97 | R5-225834 | 5409 | 1 | Correction of 6.6.2.2 Additional Spectrum Emission Mask Test configuration Table | 17.3.0 | 17.4.0 |
| 2022-09 | RAN#97 | R5-225835 | 5418 | 1 | Making test case 9.8.1.1 applicable to any UE supporting ce-ModeA | 17.3.0 | 17.4.0 |
| 2022-12 | RAN#98 | R5-226837 | 5425 | - | Correction of a typo created while introducing LTE Band 103 in spurious emission test cases in clause 6.6 | 17.4.0 | 17.5.0 |
| 2022-12 | RAN#98 | R5-226917 | 5426 | - | Updates to NB-IOT spurious emission testing | 17.4.0 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227078 | 5427 | - | Correction to lower limit of NS_56 in 6.2.4 | 17.4.0 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227887 | 5423 | 1 | Correction to General ON/OFF time mask for inter-band CA | 17.4.0 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227888 | 5428 | 1 | Correction of EARFCN | 17.4.0 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227889 | 5429 | 1 | Corrections to Reference Sensitivity for CA | 17.4.0 | 17.5.0 |
| 2022-12 | RAN#98 | R5-228050 | 5424 | 1 | Correct of Lower limit for Test Case 6.2.4_2 Additional Maximum Power Reduction (A-MPR) for UL 64QAM | 17.4.0 | 17.5.0 |
| 2023-03 | RAN#99 | R5-230098 | 5430 | - | Update the table content of TC 8.7.5.1_H.5 | 17.5.0 | 17.6.0 |
| 2023-03 | RAN#99 | R5-230099 | 5431 | - | Update the table content of TC 8.7.5.2.5 | 17.5.0 | 17.6.0 |
| 2023-03 | RAN#99 | R5-230100 | 5432 | - | Update the table content of TC 8.7.5.2_H.5 | 17.5.0 | 17.6.0 |
| 2023-03 | RAN#99 | R5-230666 | 5434 | - | Correct of format and associated sections for LTE IoT Test Cases | 17.5.0 | 17.6.0 |
| 2023-03 | RAN#99 | R5-230965 | 5436 | - | Correction to test requirements for CA_1A-42A in A-MPR test cases | 17.5.0 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231296 | 5440 | - | Update of spurious emission band UE co-existence | 17.5.0 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231297 | 5441 | - | Update of spurious emissions test case for NB-IoT | 17.5.0 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231363 | 5442 | - | Corrections on E_UTRA CA_NS_10 | 17.5.0 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231822 | 5437 | 1 | Editorial: References correction in additional spurious for CA minimum requirements | 17.5.0 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231823 | 5433 | 1 | Update Table 8.2.3.3.1.2.3-1 and Table 8.2.3.3.1.2.5-3: Minimum performance for multiple CA configurations with 3DL CCs (FRC) | 17.5.0 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231836 | 5439 | 1 | Corrections on spurious emission for UE co-existence for E-UTRA CA | 17.5.0 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231895 | 5443 | 1 | P-max definition correction for Band 14 | 17.5.0 | 17.6.0 |
| 2023-03 | RAN#99 | - | - | - | Administrative release upgrade to match the release of TS 36.508 and TS 36.521-2 which were upgraded at RAN#99 to Rel-18 due to Rel-18 relevant CR(s) | 17.6.0 | 18.0.0 |
| 2023-06 | RAN#100 | R5-232313 | 5445 | - | Introduction of LTE Band 54 to common clauses (section 5) | 18.0.0 | 18.1.0 |
| 2023-06 | RAN#100 | R5-232314 | 5446 | - | Updates to MOP and MPR test cases as part of introduction of LTE Band 54 | 18.0.0 | 18.1.0 |
| 2023-06 | RAN#100 | R5-232315 | 5447 | - | Updates to A-MPR test cases as part of introduction of LTE Band 54 | 18.0.0 | 18.1.0 |
| 2023-06 | RAN#100 | R5-232321 | 5449 | - | Updates to receiver reference sensitivity test cases as part of introduction of LTE Band 54 | 18.0.0 | 18.1.0 |
| 2023-06 | RAN#100 | R5-232322 | 5450 | - | Updates to receiver blocking test cases as part of introduction of LTE Band 54 | 18.0.0 | 18.1.0 |
| 2023-06 | RAN#100 | R5-232392 | 5452 | - | CR for 36.521-1 on p-Max corrections for Power Class 1 Band 14 | 18.0.0 | 18.1.0 |
| 2023-06 | RAN#100 | R5-232988 | 5453 | - | Correction to upper and lower limit of NS_23 in 6.2.4 | 18.0.0 | 18.1.0 |
| 2023-06 | RAN#100 | R5-232989 | 5454 | - | Correction to lower limit of NS_05 in 6.2.4A.2_1 | 18.0.0 | 18.1.0 |
| 2023-06 | RAN#100 | R5-233165 | 5456 | - | Update of UE category in test case 7.4 | 18.0.0 | 18.1.0 |
| 2023-06 | RAN#100 | R5-233546 | 5455 | 1 | Clarification of spurious emission testing configuration - LTE | 18.0.0 | 18.1.0 |
| 2023-06 | RAN#100 | R5-233704 | 5448 | 1 | Updates to spurious emissions and additional spurious emissions test cases as part of introduction of LTE Band 54 | 18.0.0 | 18.1.0 |
| 2023-09 | RAN#101 | R5-234080 | 5459 | - | Correction of test configuration for LTE inter-band DL CA and UL CA | 18.1.0 | 18.2.0 |
| 2023-09 | RAN#101 | R5-234567 | 5463 | - | Update to RefSens test cases for CA band combinations | 18.1.0 | 18.2.0 |
| 2023-09 | RAN#101 | R5-234614 | 5464 | - | Updates to Annex F3.2 as part of introduction of LTE Band 54 | 18.1.0 | 18.2.0 |
| 2023-09 | RAN#101 | R5-234900 | 5467 | - | Correction to lower limit of NS_12 in 6.2.4 | 18.1.0 | 18.2.0 |
| 2023-09 | RAN#101 | R5-235805 | 5457 | 1 | Adding test configuration and test requirement for CA_2A-2A-5A-66A,CA_2A-2A-12A-66A,CA_2A-2A-30A-66A and CA_2A-5A-66A-66A in TC7.3A.9 | 18.1.0 | 18.2.0 |
| 2023-09 | RAN#101 | R5-235810 | 5458 | 1 | Adding test configuration and test requirement for CA_5A-30A-66A-66A and CA_12A-30A-66A-66A in TC7.3A.9 | 18.1.0 | 18.2.0 |
| 2023-09 | RAN#101 | R5-235814 | 5462 | 1 | Correction of MPR requirements for UL 256QAM | 18.1.0 | 18.2.0 |
| 2023-09 | RAN#101 | R5-235815 | 5466 | 1 | Clarification of unwanted emission testing configuration - LTE | 18.1.0 | 18.2.0 |
| 2023-09 | RAN#101 | R5-235922 | 5468 | 1 | Addition of DL RMC for Band 46 BW 10 MHz | 18.1.0 | 18.2.0 |
| 2023-09 | RAN#101 | R5-235932 | 5465 | 1 | Updates to spurious emissions and additional spurious emissions test cases as part of introduction of LTE Band 54 | 18.1.0 | 18.2.0 |
| 2023-12 | RAN#102 | R5-236122 | 5469 | - | Correction of Note in Table 7.6.2F.4.1-1 | 18.2.0 | 18.3.0 |
| 2023-12 | RAN#102 | R5-236123 | 5470 | - | Correction of test configuration table number in 7.7EA.4.2 | 18.2.0 | 18.3.0 |
| 2023-12 | RAN#102 | R5-236124 | 5471 | - | Correction of some reference clause numbers | 18.2.0 | 18.3.0 |
| 2023-12 | RAN#102 | R5-236125 | 5472 | - | Introduction of measurement uncertainty for test case 7.9EA and correction of some subclause titles | 18.2.0 | 18.3.0 |
| 2023-12 | RAN#102 | R5-236399 | 5474 | - | Correct typos in Table 5.4.2A.1-x | 18.2.0 | 18.3.0 |
| 2023-12 | RAN#102 | R5-236528 | 5478 | - | Adding test requirement for CA_2A-2A-5A-30A-66A, CA_2A-2A-5A-66A-66A and CA_2A-2A-12A-30A-66A in TC7.3A.10 | 18.2.0 | 18.3.0 |
| 2023-12 | RAN#102 | R5-236550 | 5480 | - | Update to RefSens test cases 7.3A.10 for 5DL CA combo | 18.2.0 | 18.3.0 |
| 2023-12 | RAN#102 | R5-236920 | 5483 | - | Missing MU for certain IoT test cases | 18.2.0 | 18.3.0 |
| 2023-12 | RAN#102 | R5-237250 | 5486 | - | Update of Refsens CA test case | 18.2.0 | 18.3.0 |
| 2023-12 | RAN#102 | R5-237823 | 5479 | 1 | Update to RefSens test cases 7.3A.9 for 4DL CA combo | 18.2.0 | 18.3.0 |
| 2023-12 | RAN#102 | R5-237824 | 5473 | 1 | Correction the Test Configuration Table of TC7.3A.9 and TC7.3A.10 | 18.2.0 | 18.3.0 |
| 2023-12 | RAN#102 | R5-237825 | 5484 | 1 | Corrections to 5.4.2A.1 on E-UTRA CA configurations | 18.2.0 | 18.3.0 |
| 2023-12 | RAN#102 | R5-237873 | 5485 | 1 | Corrections to spurious emissions co-existence for 2 UL Inter-band CA (36.521-1) | 18.2.0 | 18.3.0 |
| 2024-03 | RAN#103 | R5-240071 | 5487 | - | Update of Annex F Measurement Uncertainties in TS 36.521-1 | 18.3.0 | 18.4.0 |
| 2024-03 | RAN#103 | R5-240614 | 5488 | - | E-UTRA V2X spurious emissions MU definition up to 26GHz | 18.3.0 | 18.4.0 |
| 2024-03 | RAN#103 | R5-240941 | 5491 | - | Update to Transmit test cases for CA_30A-48A | 18.3.0 | 18.4.0 |
| 2024-03 | RAN#103 | R5-241057 | 5499 | - | Update to freq error cat 1bis test procedure | 18.3.0 | 18.4.0 |
| 2024-03 | RAN#103 | R5-241089 | 5500 | - | Editorial correction to CA_1A-3A-7A-28A in 7.3A.9 | 18.3.0 | 18.4.0 |
| 2024-03 | RAN#103 | R5-241853 | 5490 | 1 | Update to CBW for multiples CA combos | 18.3.0 | 18.4.0 |
| 2024-03 | RAN#103 | R5-241854 | 5494 | 1 | Correct Table 7.3A.11.4.1-1 of CA Configurations | 18.3.0 | 18.4.0 |
| 2024-03 | RAN#103 | R5-241855 | 5495 | 1 | Correct Table 7.4A.8.4.1-2 & Table 7.4A.8_H.4.1-2 & Table 7.4A.9.4.1-1 of CA Configurations | 18.3.0 | 18.4.0 |
| 2024-03 | RAN#103 | R5-241856 | 5496 | 1 | Correct Table 7.5A.8.4.1-1 & Table 7.5A.9.4.1-1 & Table 7.6.1A.8.4.1-1 of CA Configurations | 18.3.0 | 18.4.0 |
| 2024-03 | RAN#103 | R5-241857 | 5497 | 1 | Correct typos in Table 7.3A.5.4.1-1 | 18.3.0 | 18.4.0 |
| 2024-03 | RAN#103 | R5-241874 | 5492 | 1 | Update to Receiver test cases for multiple CA combos | 18.3.0 | 18.4.0 |
| 2024-03 | RAN#103 | R5-241925 | 5493 | 1 | Update Table notes in TS36.521-1 TC-7.6.3EA Table-7.6.3EA.3-1: Narrow-band blocking | 18.3.0 | 18.4.0 |
| 2024-06 | RAN#104 | R5-242283 | 5503 | - | Editorial Correction of test configuration table of Reference sensitivity level for 4DL CA | 18.4.0 | 18.5.0 |
| 2024-06 | RAN#104 | R5-242386 | 5504 | - | Addition of band 106 into frequency bands and channel arrangements | 18.4.0 | 18.5.0 |
| 2024-06 | RAN#104 | R5-242387 | 5505 | - | Addition of band 106 transmitter requirements | 18.4.0 | 18.5.0 |
| 2024-06 | RAN#104 | R5-242388 | 5506 | - | Addition of band 106 receiver requirements | 18.4.0 | 18.5.0 |
| 2024-06 | RAN#104 | R5-242421 | 5507 | - | Editorial correction to the RMC of 8.12.1.1.2 | 18.4.0 | 18.5.0 |
| 2024-06 | RAN#104 | R5-242663 | 5508 | - | Update Test Configuration for NS17 Band 28 3MHz | 18.4.0 | 18.5.0 |
| 2024-06 | RAN#104 | R5-242701 | 5509 | - | Addition of Inter-band DL CA and CA_30A-48A band for A-MPR & Spurious emission test cases | 18.4.0 | 18.5.0 |
| 2024-06 | RAN#104 | R5-242732 | 5511 | - | Addition of MOP requirements for 2UL CA 3A-42A | 18.4.0 | 18.5.0 |
| 2024-06 | RAN#104 | R5-242782 | 5512 | - | Update of Receiver test cases for multiple CA combos | 18.4.0 | 18.5.0 |
| 2024-06 | RAN#104 | R5-243005 | 5514 | - | Correction to test requirement of CA_2A-2A-66A-66A in 7.3A.9 | 18.4.0 | 18.5.0 |
| 2024-06 | RAN#104 | R5-243006 | 5515 | - | Correction to test requirement of CA_2A-2A-12A-66A-66A in 7.3A.10 | 18.4.0 | 18.5.0 |
| 2024-06 | RAN#104 | R5-243807 | 5502 | 1 | Correction of Additional spurious emission test requirement for NS_45 | 18.4.0 | 18.5.0 |
| 2024-06 | RAN#104 | R5-243808 | 5516 | 1 | Correction of narrow band blocking for CA | 18.4.0 | 18.5.0 |
| 2024-09 | RAN#105 | R5-244687 | 5518 | - | Update of CA configurations in TC8.2.3.1.1.2 | 18.5.0 | 18.6.0 |
| 2024-09 | RAN#105 | R5-244811 | 5519 | - | Clarification about order of SCCs in 5CA Rx test cases | 18.5.0 | 18.6.0 |
| 2024-09 | RAN#105 | R5-244900 | 5520 | - | Update of reference sensitivity CA test cases | 18.5.0 | 18.6.0 |
| 2024-09 | RAN#105 | R5-244904 | 5521 | - | Correction of test case 8.2.3.2.2A | 18.5.0 | 18.6.0 |
| 2024-09 | RAN#105 | R5-245076 | 5522 | - | Update to MOP test case and remove band 47 requirement | 18.5.0 | 18.6.0 |
| 2024-09 | RAN#105 | R5-245982 | 5517 | 1 | Correction of test case 6.2.4A.1_3 A-MPR for CA HPUE (CA_NS_04) | 18.5.0 | 18.6.0 |
| 2024-12 | RAN#106 | R5-246265 | 5523 | - | Addition of PC1 operation for bands 40, 42 and 106 | 18.6.0 | 18.7.0 |
| 2024-12 | RAN#106 | R5-246266 | 5524 | - | Addition of PC1 operation for band 12 | 18.6.0 | 18.7.0 |
| 2024-12 | RAN#106 | R5-247058 | 5526 | - | Update to RefSens test case for muliple 4CA and 5CA configs | 18.6.0 | 18.7.0 |
| 2024-12 | RAN#106 | R5-247190 | 5527 | - | Addition of 3 MHz test requirement for NS_17 in 6.2.4 | 18.6.0 | 18.7.0 |
| 2024-12 | RAN#106 | R5-247191 | 5528 | - | Clarification of test condition in 3DL CA Rx test cases | 18.6.0 | 18.7.0 |
| 2024-12 | RAN#106 | R5-247831 | 5529 | 1 | Corrections on spurious emission band UE co-existence for CA | 18.6.0 | 18.7.0 |
| 2024-12 | RAN#106 | R5-247832 | 5525 | 1 | Correction of frequency range in Test Purpose for V2X blocking test cases | 18.6.0 | 18.7.0 |
| 2025-03 | RAN#107 | R5-250260 | 5531 | - | Correction to A-MPR for NS_06 PC1 in band 12 for 64QAM and 256QAM | 18.7.0 | 18.8.0 |
| 2025-03 | RAN#107 | R5-250563 | 5533 | - | Addition of new bands, bandwidth for LTE based 5G broadcast | 18.7.0 | 18.8.0 |
| 2025-03 | RAN#107 | R5-250564 | 5534 | - | Addition of PMCH decoding performance test cases for LTE based 5G broadcast | 18.7.0 | 18.8.0 |
| 2025-03 | RAN#107 | R5-250943 | 5535 | - | Correction of statistical testing of receiver characteristics for LTE | 18.7.0 | 18.8.0 |
| 2025-03 | RAN#107 | R5-251102 | 5536 | - | Update 6.6.3.2A for removal of CA BW class for spurious emission band UE co-existence | 18.7.0 | 18.8.0 |
| 2025-03 | RAN#107 | R5-251672 | 5537 | 1 | Correction of RefSens CA test cases | 18.7.0 | 18.8.0 |
| 2025-06 | RAN#108 | R5-252059 | 5538 | - | Correction to test configuration of NS_05 in Clause 6.6.3.3A | 18.8.0 | 18.9.0 |
| 2025-06 | RAN#108 | R5-252849 | 5542 | - | Corrections on 6.2.2_1 for HPUE power class for band 40 and 42 | 18.8.0 | 18.9.0 |
| 2025-06 | RAN#108 | R5-253645 | 5541 | 1 | Adding missing UTRA_ACLR applicablity for ACLR requirements for PC3 | 18.8.0 | 18.9.0 |