Annex A (normative):
Reference Measurement Channels

# A.1	PDSCH

## A.1.1	FDD

Table A.1.1-1: PDSCH Reference Measurement Channels for FDD

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.2 FDD |  | R.5 FDD | R.7 FDD | R.0 FDD | R.1 FDD | R.3 FDD | R.4 FDD | R.6 FDD |
| Channel bandwidth | MHz | 1.4 | 3 | 5 | 5 | 10 | 10 | 10 | 20 | 20 |
| Number of transmitter antennas |  | 1 |  | 1 | 1 | 1 | 2 | 1 | 1 | 1 |
| Allocated resource blocks (Note 4) |  | 2 |  | 11 | 11 | 24 | 24 | 24 | 24 | 24 |
| Allocated subframes per Radio Frame |  | 10 |  | 10 | 10 | 10 | 10 | 10 | 10 | 10 |
| Modulation |  | QPSK |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |  |  |  |  |  |  |  |
| For Sub-Frames 4, 9 | Bits | 120 |  | 968 | 968 | 2088 | 2088 | 2088 | 2088 | 2088 |
| For Sub-Frame 5 | Bits | 104 |  | 776 | 776 | 2088 | 1736 | 2088 | 2088 | 2088 |
| For Sub-Frame 0 | Bits | 32 |  | 616 | 616 | 1736 | 1736 | 1736 | 1736 | 1736 |
| For Sub-Frame 1, 2, 3, 6, 7, 8 | Bits | 0 |  | 0 | 968 | 0 | 0 | 2088 | 0 | 2088 |
| Number of Code Blocks per Sub-Frame (Note 5) |  |  |  |  |  |  |  |  |  |  |
| For Sub-Frames 4, 9 |  | 1 |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| For Sub-Frame 5 |  | 1 |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| For Sub-Frame 0 |  | 1 |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| For Sub-Frame 1, 2, 3, 6, 7, 8 |  | 0 |  | 0 | 1 | 0 | 0 | 1 | 0 | 1 |
| Binary Channel Bits Per Sub-Frame |  |  |  |  |  |  |  |  |  |  |
| For Sub-Frames 4, 9 | Bits | 456 |  | 2772 | 2772 | 6624 | 6336 | 6624 | 6624 | 6624 |
| For Sub-Frame 5 | Bits | 360 |  | 2484 | 2484 | 6336 | 6048 | 6336 | 6336 | 6336 |
| For Sub-Frame 0 | Bits | 176 |  | 1932 | 1932 | 5784 | 5520 | 5784 | 5784 | 5784 |
| For Sub-Frame 1, 2, 3, 6, 7, 8 | Bits | 0 |  | 0 | 2772 | 0 | 0 | 6624 | 0 | 6624 |
| Max. Throughput averaged over 1 frame | kbps | 37.6 |  | 332.8 | 913.6 | 800 | 765 | 2053 | 800 | 2053 |
| NOTE 1: 2 symbols allocated to PDCCH for 10 MHz channel BW. 4 symbols allocated to PDCCH for 1.4 MHz channel BW. 3 OFDM symbols allocated to PDCCH for 5 MHz channel BW. NOTE 2: Reference signal, synchronization signals and PBCH allocated as defined in 3GPP TS 36.211 [9]. NOTE 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 36.213 [8]. NOTE 4: Allocation is located in the middle of bandwidth. NOTE 5: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) NOTE 6: PDSCH allocation applies only to subframes not configured as PRS subframes. |  |  |  |  |  |  |  |  |  |  |

Table A.1.1-2: PDSCH Reference Measurement Channels for FDD with slot duration TTI

| Parameter | Unit | R.8 FDD |
| --- | --- | --- |
| Bandwidth | MHz | 10 |
| Number of transmit antennas |  | 1 |
| Allocated resource blocks(Note 4) |  | 24 |
| Allocated subframes per Radio Frame |  | 10 |
| Modulation |  | QPSK |
| Target coding rate |  | 1/3 |
| Information Bit Payload |  |  |
| For Sub-Frame 4, 9 |  |  |
| Slot index 0 | Bits | 872 |
| Slot index 1 | Bits | 1224 |
| For Sub-Frame 5 |  |  |
| Slot index 0 | Bits | 872 |
| Slot index 1 | Bits | 1224 |
| For Sub-Frame 0 |  |  |
| Slot index 0 | Bits | 872 |
| Slot index 1 | Bits | 1032 |
| For Sub-Frame 1, 2, 3, 6, 7, 8 |  |  |
| Slot index 0 | Bits | 0 |
| Slot index 1 | Bits | 0 |
| Number of Code Blocks per slot (Note 5) |  |  |
| For Sub-Frame 4, 9 |  |  |
| Slot index 0 |  | 1 |
| Slot index 1 |  | 1 |
| For Sub-Frame 5 |  |  |
| Slot index 0 |  | 1 |
| Slot index 1 |  | 1 |
| For Sub-Frame 0 |  |  |
| Slot index 0 |  | 1 |
| Slot index 1 |  | 1 |
| For Sub-Frame 1, 2, 3, 6, 7, 8 |  |  |
| Slot index 0 |  | 0 |
| Slot index 1 |  | 0 |
| Binary Channel Bits per slot |  |  |
| For Sub-Frame 4, 9 | Bits |  |
| Slot index 0 | Bits | 2784 |
| Slot index 1 | Bits | 3840 |
| For Sub-Frame 5 | Bits |  |
| Slot index 0 | Bits | 2496 |
| Slot index 1 | Bits | 3840 |
| For Sub-Frame 0 | Bits |  |
| Slot index 0 | Bits | 2496 |
| Slot index 1 | Bits | 3288 |
| For Sub-Frame 1, 2, 3, 6, 7, 8 | Bits |  |
| Slot index 0 | Bits | 0 |
| Slot index 1 | Bits | 0 |
| Max. Throughput averaged over 1 frame | kbps | 819.2 |
| NOTE 1: 2 symbols allocated to PDCCH for 10 MHz channel BW. 4 symbols allocated to PDCCH for 1.4 MHz channel BW. 3 OFDM symbols allocated to PDCCH for 5 MHz channel BW. NOTE 2: Reference signal, synchronization signals and PBCH allocated as defined in 3GPP TS 36.211 [9]. NOTE 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 36.213 [8]. NOTE 4: Allocation is located in the middle of bandwidth. NOTE 5: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 6: PDSCH allocation applies only to subframes not configured as PRS subframes. NOTE 7: Note SPDCCH is transmitted outside allocated resource blocks. |  |  |

Table A.1.1-3: PDSCH Reference Measurement Channels for FDD with subslot duration TTI

| Parameter | Unit | R.9 FDD |
| --- | --- | --- |
| Bandwidth | MHz | 10 |
| Number of transmit antennas |  | 1 |
| Allocated resource blocks(Note 4) |  | 24 |
| Allocated subframes per Radio Frame |  | 10 |
| Modulation |  | QPSK |
| Target coding rate |  | 1/3 |
| Information Bit Payload |  |  |
| For Sub-Frame 4, 9 |  |  |
| Subslot index 1 | Bits | 504 |
| Subslot index 2 | Bits | 344 |
| Subslot index 3 | Bits | 344 |
| Subslot index 4 | Bits | 344 |
| Subslot index 5 | Bits | 504 |
| For Sub-Frame 5 |  |  |
| Subslot index 1 | Bits | 504 |
| Subslot index 2 | Bits | 288 |
| Subslot index 3 | Bits | 344 |
| Subslot index 4 | Bits | 344 |
| Subslot index 5 | Bits | 504 |
| For Sub-Frame 0 |  |  |
| Subslot index 1 | Bits | 504 |
| Subslot index 2 | Bits | 288 |
| Subslot index 3 | Bits | 224 |
| Subslot index 4 | Bits | 288 |
| Subslot index 5 |  | 504 |
| For Sub-Frame 1, 2, 3, 6, 7, 8 |  |  |
| Subslot index 1 | Bits | 0 |
| Subslot index 2 | Bits | 0 |
| Subslot index 3 | Bits | 0 |
| Subslot index 4 | Bits | 0 |
| Subslot index 5 | Bits | 0 |
| Number of Code Blocks per Sub-Frame(Note 5) |  |  |
| For Sub-Frame 4, 9 |  |  |
| Subslot index 1 |  | 1 |
| Subslot index 2 |  | 1 |
| Subslot index 3 |  | 1 |
| Subslot index 4 |  | 1 |
| Subslot index 5 |  | 1 |
| For Sub-Frame 5 |  |  |
| Subslot index 1 |  | 1 |
| Subslot index 2 |  | 1 |
| Subslot index 3 |  | 1 |
| Subslot index 4 |  | 1 |
| Subslot index 5 |  | 1 |
| For Sub-Frame 0 |  |  |
| Subslot index 1 |  | 1 |
| Subslot index 2 |  | 1 |
| Subslot index 3 |  | 1 |
| Subslot index 4 |  | 1 |
| Subslot index 5 |  | 1 |
| For Sub-Frame 1, 2, 3, 6, 7, 8 |  |  |
| Subslot index 1 |  | 0 |
| Subslot index 2 |  | 0 |
| Subslot index 3 |  | 0 |
| Subslot index 4 |  | 0 |
| Subslot index 5 |  | 0 |
| Binary Channel Bits per subslot |  |  |
| For Sub-Frame 4, 9 | Bits |  |
| Subslot index 1 | Bits | 1632 |
| Subslot index 2 | Bits | 1152 |
| Subslot index 3 | Bits | 1056 |
| Subslot index 4 | Bits | 1152 |
| Subslot index 5 | Bits | 1632 |
| For Sub-Frame 5 | Bits |  |
| Subslot index 1 | Bits | 1632 |
| Subslot index 2 | Bits | 864 |
| Subslot index 3 | Bits | 1056 |
| Subslot index 4 | Bits | 1152 |
| Subslot index 5 | Bits | 1632 |
| For Sub-Frame 0 | Bits |  |
| Subslot index 1 | Bits | 1632 |
| Subslot index 2 | Bits | 864 |
| Subslot index 3 | Bits | 792 |
| Subslot index 4 | Bits | 864 |
| Subslot index 5 | Bits | 1632 |
| For Sub-Frame 1, 2, 3, 6, 7, 8 | Bits |  |
| Subslot index 1 | Bits | 0 |
| Subslot index 2 | Bits | 0 |
| Subslot index 3 | Bits | 0 |
| Subslot index 4 | Bits | 0 |
| Subslot index 5 | Bits | 0 |
| Max. Throughput averaged over 1 frame | kbps | 787.2 |
| NOTE 1: 2 symbols allocated to PDCCH for 10 MHz channel BW. 4 symbols allocated to PDCCH for 1.4 MHz channel BW. 3 OFDM symbols allocated to PDCCH for 5 MHz channel BW. NOTE 2: Reference signal, synchronization signals and PBCH allocated as defined in 3GPP TS 36.211 [9]. NOTE 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 36.213 [8]. NOTE 4: Allocation is located in the middle of bandwidth. NOTE 5: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 6: PDSCH allocation applies only to subframes not configured as PRS subframes. NOTE 7: Note SPDCCH is transmitted outside allocated resource blocks. |  |  |

## A.1.2	TDD

Table A.1.2-1: PDSCH Reference Measurement Channels for TDD UL/DL configuration1

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.2 TDD |  | R.4 TDD | R.0 TDD | R.1 TDD | R.3 TDD |
| Channel bandwidth | MHz | 1.4 | 3 | 5 | 10 | 10 | 20 |
| Number of transmitter antennas |  | 1 |  | 1 | 1 | 2 | 1 |
| Allocated resource blocks (Note 4) |  | 2 |  | 11 | 24 | 24 | 24 |
| Uplink-Downlink Configuration (Note 5) |  | 1 |  | 1 | 1 | 1 | 1 |
| Special Subframe Configuration (Note 6) |  | 6 |  | 6 | 6 | 6 | 6 |
| Allocated subframes per Radio Frame |  | 6 |  | 6 | 6 | 6 | 6 |
| Modulation |  | QPSK |  | QPSK | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 |  | 1/3 | 1/3 | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |  |  |  |  |
| For Sub-Frames 4,9 | Bits | 120 |  | 968 | 2088 | 2088 | 2088 |
| For Sub-Frame 5 | Bits | 104 |  | 968 | 2088 | 2088 | 2088 |
| For Sub-Frame 0 | Bits | 56 |  | 616 | 2088 | 1736 | 2088 |
| For Sub-Frame 1, 6 (DwPTS) | Bits | 56 |  | 552 | 1032 | 1032 | 1032 |
| Number of Code Blocks per Sub-Frame (Note 7) |  |  |  | 1 |  |  | 1 |
| For Sub-Frames 4,9 |  | 1 |  | 1 | 1 | 1 | 1 |
| For Sub-Frame 5 |  | 1 |  | 1 | 1 | 1 | 1 |
| For Sub-Frame 0 |  | 1 |  | 1 | 1 | 1 | 1 |
| For Sub-Frame 1, 6 (DwPTS) |  | 1 |  | 1 | 1 | 1 | 1 |
| Binary Channel Bits Per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frames 4,9 | Bits | 456 |  | 2772 | 6624 | 6336 | 6624 |
| For Sub-Frame 5 | Bits | 408 |  | 2628 | 6480 | 6192 | 6480 |
| For Sub-Frame 0 | Bits | 224 |  | 2076 | 5928 | 5664 | 5928 |
| For Sub-Frame 1, 6 (DwPTS) | Bits | 272 |  | 1616 | 3696 | 3504 | 3696 |
| Max. Throughput averaged over 1 frame | Mbps | 0.05612 |  | 0.4624 | 1.0416 | 1.0064 | 1.0416 |
| NOTE 1: 2 symbols allocated to PDCCH for 10 MHz channel BW. 3 OFDM symbols allocated to PDCCH for 5MHz channel BW. 4 symbols allocated to PDCCH for 1.4 MHz channel BW. For special subframe (1 & 6) only 2 OFDM symbols are allocated to PDCCH for all bandwidths. NOTE 2: Reference signal, synchronization signals and PBCH allocated as defined in 3GPP TS 36.211 [9].4 symbols allocated to PDCCH for 1.4 MHz channel BW. NOTE 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 36.213 [8].Reference signal, synchronization signals and PBCH allocated as defined in 3GPP TS 36.211 [9]. NOTE 4: Allocation is located in the middle of bandwidth. If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 36.213 [8]. NOTE 5: As per Table 4.2-2 in 3GPP TS 36.211 [9]. NOTE 6: As per Table 4.2-1 in 3GPP TS 36.211 [9]. NOTE 7: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 8: PDSCH allocation applies only to subframes not configured as PRS subframes. |  |  |  |  |  |  |  |

Table A.1.2-2: PDSCH Reference Measurement Channels for TDD UL/DL configuration0

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  |  |  |  | R.5 TDD |  |  |
| Channel bandwidth | MHz | 1.4 | 3 | 5 | 10 | 10 | 20 |
| Number of transmitter antennas |  |  |  |  | 1 |  |  |
| Allocated resource blocks (Note 4) |  |  |  |  | 24 |  |  |
| Uplink-Downlink Configuration (Note 5) |  |  |  |  | 0 |  |  |
| Special Subframe Configuration (Note 6) |  |  |  |  | 6 |  |  |
| Allocated subframes per Radio Frame |  |  |  |  | 4 |  |  |
| Modulation |  |  |  |  | QPSK |  |  |
| Target Coding Rate |  |  |  |  | 1/3 |  |  |
| Information Bit Payload |  |  |  |  |  |  |  |
| For Sub-Frames 4,9 | Bits |  |  |  | N/A |  |  |
| For Sub-Frame 5 | Bits |  |  |  | 2088 |  |  |
| For Sub-Frame 0 | Bits |  |  |  | 2088 |  |  |
| For Sub-Frame 1, 6 (DwPTS) | Bits |  |  |  | 1032 |  |  |
| Number of Code Blocks per Sub-Frame (Note 7) |  |  |  |  | 1 |  |  |
| For Sub-Frames 4,9 |  |  |  |  | N/A |  |  |
| For Sub-Frame 5 |  |  |  |  | 1 |  |  |
| For Sub-Frame 0 |  |  |  |  | 1 |  |  |
| For Sub-Frame 1, 6 (DwPTS) |  |  |  |  | 1 |  |  |
| Binary Channel Bits Per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frames 4,9 | Bits |  |  |  | N/A |  |  |
| For Sub-Frame 5 | Bits |  |  |  | 6480 |  |  |
| For Sub-Frame 0 | Bits |  |  |  | 5928 |  |  |
| For Sub-Frame 1, 6 (DwPTS) | Bits |  |  |  | 3696 |  |  |
| Max. Throughput averaged over 1 frame | Mbps |  |  |  | 0.624 |  |  |
| NOTE 1: 2 symbols allocated to PDCCH for 10 MHz channel BW. 3 OFDM symbols allocated to PDCCH for 5MHz channel BW. 4 symbols allocated to PDCCH for 1.4 MHz channel BW. For special subframe (1 & 6) only 2 OFDM symbols are allocated to PDCCH for all bandwidths. NOTE 2: Reference signal, synchronization signals and PBCH allocated as defined in 3GPP TS 36.211 [9]. NOTE 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 36.213 [8]. NOTE 4: Allocation is located in the middle of bandwidth. NOTE 5: As per Table 4.2-2 in 3GPP TS 36.211 [9]. NOTE 6: As per Table 4.2-1 in 3GPP TS 36.211 [9]. NOTE 7: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 8: PDSCH allocation applies only to subframes not configured as PRS subframes. |  |  |  |  |  |  |  |

Table A.1.2-3: PDSCH Reference Measurement Channels for TDD slot duration TTI

| Parameter | Unit | R.6 TDD |
| --- | --- | --- |
| Bandwidth | MHz | 10 |
| Number of transmit antennas |  | 1 |
| Allocated resource blocks(Note 4) |  | 24 |
| Uplink-Downlink Configuration (Note 5) |  | 1 |
| Special Subframe Configuration (Note 6) |  | 6 |
| Allocated subframes per Radio Frame |  | 6 |
| Modulation |  | QPSK |
| Target coding rate |  | 1/3 |
| Information Bit Payload |  |  |
| For Sub-Frame 4, 9 |  |  |
| Slot index 0 | Bits | 872 |
| Slot index 1 | Bits | 1224 |
| For Sub-Frame 5 |  |  |
| Slot index 0 | Bits | 872 |
| Slot index 1 | Bits | 1224 |
| For Sub-Frame 0 |  |  |
| Slot index 0 | Bits | 872 |
| Slot index 1 | Bits | 1032 |
| For Sub-Frame 1, 6 (DwPTS) |  | (1032) |
| Slot index 0 | Bits | 872 |
| Slot index 1 | Bits | N/A |
| Number of Code Blocks per Sub-Frame |  |  |
| For Sub-Frame 4, 9 |  |  |
| Slot index 0 |  | 1 |
| Slot index 1 |  | 1 |
| For Sub-Frame 5 |  |  |
| Slot index 0 |  | 1 |
| Slot index 1 |  | 1 |
| For Sub-Frame 0 |  |  |
| Slot index 0 |  | 1 |
| Slot index 1 |  | 1 |
| For Sub-Frame 1, 6 (DwPTS) |  |  |
| Slot index 0 |  | 1 |
| Slot index 1 |  | N/A |
| Binary Channel Bits per slot |  |  |
| For Sub-Frame 4, 9 | Bits |  |
| Slot index 0 | Bits | 2784 |
| Slot index 1 | Bits | 3840 |
| For Sub-Frame 5 | Bits |  |
| Slot index 0 | Bits | 2784 |
| Slot index 1 | Bits | 3696 |
| For Sub-Frame 0 | Bits |  |
| Slot index 0 | Bits | 2640 |
| Slot index 1 | Bits | 3288 |
| For Sub-Frame 1, 6 (DwPTS) | Bits |  |
| Slot index 0 | Bits | 2640 |
| Slot index 1 | Bits | N/A |
| Max. Throughput averaged over 1 frame | kbps | 999.6 |
| NOTE 1: 2 symbols allocated to PDCCH for 10 MHz channel BW. 3 OFDM symbols allocated to PDCCH for 5MHz channel BW. 4 symbols allocated to PDCCH for 1.4 MHz channel BW. For special subframe (1 & 6) only 2 OFDM symbols are allocated to PDCCH for all bandwidths. NOTE 2: Reference signal, synchronization signals and PBCH allocated as defined in 3GPP TS 36.211 [9]. NOTE 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 36.213 [8]. NOTE 4: Allocation is located in the middle of bandwidth. NOTE 5: As per Table 4.2-2 in 3GPP TS 36.211 [9]. NOTE 6: As per Table 4.2-1 in 3GPP TS 36.211 [9]. NOTE 7: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 8: PDSCH allocation applies only to subframes not configured as PRS subframes. NOTE 9: SPDCCH is transmitted outside allocated resource blocks. |  |  |

## A.1.3	FDD for UE Category 0

Table A.1.3-1: PDSCH Reference Measurement Channels for FDD

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | R.13 FDD | R.14 FDD | R.15 FDD |
| Channel bandwidth | MHz | 10 | 10 | 10 |
| Number of transmitter antennas |  | 1 | 2 | 2 |
| Allocated resource blocks (Note 4) |  | 24 | 24 | 24 |
| Allocated subframes per Radio Frame |  | 10 | 10 | 10 |
| Modulation |  | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/10 | 1/10 | 1/10 |
| Information Bit Payload |  |  |  |  |
| For Sub-Frames 4, 9 | Bits | 648 | 648 | 648 |
| For Sub-Frame 5 | Bits | 648 | 648 | 648 |
| For Sub-Frame 0 | Bits | 648 | 648 | 648 |
| For Sub-Frame 1, 2, 3, 6, 7, 8 | Bits | 0 | 0 | 648 |
| Number of Code Blocks per Sub-Frame (Note 5) |  |  |  |  |
| For Sub-Frames 4, 9 |  | 1 | 1 | 1 |
| For Sub-Frame 5 |  | 1 | 1 | 1 |
| For Sub-Frame 0 |  | 1 | 1 | 1 |
| For Sub-Frame 1, 2, 3, 6, 7, 8 |  | 1 | 1 | 1 |
| Binary Channel Bits Per Sub-Frame |  |  |  |  |
| For Sub-Frames 4, 9 | Bits | 6624 | 6336 | 6636 |
| For Sub-Frame 5 | Bits | 6336 | 6048 | 6408 |
| For Sub-Frame 0 | Bits | 5784 | 5520 | 5520 |
| For Sub-Frame 1, 2, 3, 6, 7, 8 | Bits | 0 | 0 | 6636 |
| Max. Throughput averaged over 1 frame | kbps | 259.2 | 259.2 | 648 |
| NOTE 1: 2 symbols allocated to PDCCH for 10 MHz channel BW. 4 symbols allocated to PDCCH for 1.4 MHz channel BW. 3 OFDM symbols allocated to PDCCH for 5 MHz channel BW. NOTE 2: Reference signal, synchronization signals and PBCH allocated as defined in 3GPP TS 36.211 [9]. NOTE 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 36.213 [8]. NOTE 4: Allocation is located in the middle of bandwidth. NOTE 5: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 6: PDSCH allocation applies only to subframes not configured as PRS subframes. |  |  |  |  |

## A.1.4	HD-FDD for UE Category 0

Table A.1.4-1: PDSCH Reference Measurement Channels for HD-FDD

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | R.1 HD-FDD | R.2 HD-FDD |
| Channel bandwidth | MHz | 10 | 10 |
| Number of transmitter antennas |  | 1 | 2 |
| Allocated resource blocks (Note 4) |  | 24 | 24 |
| Allocated subframes per Radio Frame |  | 10 | 10 |
| Modulation |  | QPSK | QPSK |
| Target Coding Rate |  | 1/10 | 1/10 |
| Information Bit Payload |  |  |  |
| For Sub-Frames 4, 9, | Bits | 0 | 0 |
| For Sub-Frame 5 (Note 7) | Bits | 424 | 424 |
| For Sub-Frame 0 (Note 7) |  | 648 | 648 |
| For Sub-Frame 1, 2, 3, 6, 7, 8 | Bits | 0 | 0 |
| For Sub-Frame 1, 6 | Bits |  |  |
| Number of Code Blocks per Sub-Frame (Note 5) |  |  |  |
| For Sub-Frames 4, 9 |  | 1 | 1 |
| For Sub-Frame 5 |  | 1 | 1 |
| For Sub-Frame 0 |  | 1 | 1 |
| For Sub-Frame 1, 2, 3, 6, 7, 8 |  | 1 | 1 |
| Binary Channel Bits Per Sub-Frame |  |  |  |
| For Sub-Frames 4, 9 | Bits | 0 | 0 |
| For Sub-Frame 5 | Bits | 6336 | 6048 |
| For Sub-Frame 0 | Bits | 5784 | 5520 |
| For Sub-Frame 1, 2, 3, 6, 7, 8 | Bits | 0 | 0 |
| Max. Throughput averaged over 1 frame | kbps |  |  |
| NOTE 1: 2 symbols allocated to PDCCH for 10 MHz channel BW. 4 symbols allocated to PDCCH for 1.4 MHz channel BW. 3 OFDM symbols allocated to PDCCH for 5 MHz channel BW. NOTE 2: Reference signal, synchronization signals and PBCH allocated as defined in 3GPP TS 36.211 [9]. NOTE 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 36.213 [8]. NOTE 4: Allocation is located in the middle of bandwidth. NOTE 5: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 6: PDSCH allocation applies only to subframes not configured as PRS subframes. NOTE 7: Sub-frame 0 or sub-frame 5 is randomly scheduled in downlink. |  |  |  |

## A.1.5	TDD for UE Category 0

Table A.1.5-1: PDSCH Reference Measurement Channels for TDD UL/DL configuration1

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | R.12 TDD | R.13 TDD |
| Channel bandwidth | MHz | 10 | 10 |
| Number of transmitter antennas |  | 1 | 2 |
| Allocated resource blocks (Note 4) |  | 24 | 24 |
| Uplink-Downlink Configuration (Note 5) |  | 1 | 1 |
| Special Subframe Configuration (Note 6) |  | 6 | 6 |
| Allocated subframes per Radio Frame |  | 6 | 6 |
| Modulation |  | QPSK | QPSK |
| Target Coding Rate |  | 1/10 | 1/10 |
| Information Bit Payload |  |  |  |
| For Sub-Frames 4,9 | Bits | 648 | 648 |
| For Sub-Frame 5 | Bits | 648 | 648 |
| For Sub-Frame 0 | Bits | 648 | 648 |
| For Sub-Frame 1, 6 (DwPTS) | Bits | 488 | 488 |
| Number of Code Blocks per Sub-Frame (Note 7) |  | 1 | 1 |
| For Sub-Frames 4,9 |  | 1 | 1 |
| For Sub-Frame 5 |  | 1 | 1 |
| For Sub-Frame 0 |  | 1 | 1 |
| For Sub-Frame 1, 6 (DwPTS) |  | 1 | 1 |
| Binary Channel Bits Per Sub-Frame |  |  |  |
| For Sub-Frames 4,9 | Bits | 6624 | 6336 |
| For Sub-Frame 5 | Bits | 6580 | 6192 |
| For Sub-Frame 0 | Bits | 5928 | 5664 |
| For Sub-Frame 1, 6 (DwPTS) | Bits | 3696 | 3408 |
| Max. Throughput averaged over 1 frame | Mbps | 0.3552 | 0.3552 |
| NOTE 1: 2 symbols allocated to PDCCH for 10 MHz channel BW. 3 OFDM symbols allocated to PDCCH for 5MHz channel BW. 4 symbols allocated to PDCCH for 1.4 MHz channel BW. For special subframe (1 & 6) only 2 OFDM symbols are allocated to PDCCH for all bandwidths. NOTE 2: Reference signal, synchronization signals and PBCH allocated as defined in 3GPP TS 36.211 [9]. NOTE 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 36.213 [8]. NOTE 4: Allocation is located in the middle of bandwidth. NOTE 5: As per Table 4.2-2 in 3GPP TS 36.211 [9]. NOTE 6: As per Table 4.2-1 in 3GPP TS 36.211 [9]. NOTE 7: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 8: PDSCH allocation applies only to subframes not configured as PRS subframes. |  |  |  |

## A.1.6	Frame Structure 3

Table A.1.6-1: PDSCH Reference Measurement Channels for FS 3

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  |  |  |  | R.0 FS3 | R.1 FS3 |
| Channel bandwidth | MHz |  |  |  | 20 | 20 |
| Number of transmitter antennas |  |  |  |  | 1 | 1 |
| Allocated resource blocks (Note 4) |  |  |  |  | 24 | 24 |
| Allocated subframes per Radio Frame |  |  |  |  | 10 | 10 |
| Modulation |  |  |  |  | QPSK | QPSK |
| Target Coding Rate |  |  |  |  | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |  |  |  |
| For Sub-Frames 4, 9 | Bits |  |  |  | 2088 | 2088 |
| For Sub-Frame 5 | Bits |  |  |  | 2088 | 2088 |
| For Sub-Frame 0 | Bits |  |  |  | 2088 | 2088 (Note 7) |
| For Sub-Frame 1, 2, 3, 6, 7, 8 | Bits |  |  |  | 2088 | 0 |
| Number of Code Blocks per Sub-Frame (Note 5) |  |  |  |  |  |  |
| For Sub-Frames 4, 9 |  |  |  |  | 1 | 1 |
| For Sub-Frame 5 |  |  |  |  | 1 | 1 |
| For Sub-Frame 0 |  |  |  |  | 1 | 1 |
| For Sub-Frame 1, 2, 3, 6, 7, 8 |  |  |  |  | 1 | 1 |
| Binary Channel Bits Per Sub-Frame |  |  |  |  |  |  |
| For Sub-Frames 4, 9 | Bits |  |  |  | 6624 | 6624 |
| For Sub-Frame 5 | Bits |  |  |  | 6336 | 6336 |
| For Sub-Frame 0 | Bits |  |  |  | 6336 | 6336 |
| For Sub-Frame 1, 2, 3, 6, 7, 8 | Bits |  |  |  | 6624 | 0 |
| Max. Throughput averaged over 1 frame | kbps |  |  |  | 2088 | 2088 |
| NOTE 1: 2 symbols allocated to PDCCH for 20 MHz channel BW. NOTE 2: Reference signal, synchronization signals allocated as defined in 3GPP TS 36.211 [9]. NOTE 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 36.213 [8]. NOTE 4: Allocation is located in the middle of bandwidth. NOTE 5: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 6: PDSCH allocation applies only to subframes not configured as PRS subframes. NOTE 7: PDSCH allocation applies only to subframes where there is no DRS transmission NOTE 8: PDSCH is not transmitted in subframes where it is determined that transmission should not occur according to the listen before talk (LBT model). NOTE 9: Max throughput averaged over 1 frame does not account for missed PDSCH transmission due to LBT or DRS transmission. |  |  |  |  |  |  |

# A.2	PCFICH/PDCCH/PHICH

## A.2.1	FDD

Table A.2.1-1: PCFICH/PDCCH/PHICH Reference Channel for FDD

| Parameter | Unit | Value |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.8 FDD | R.11 FDD | R.12 FDD | R.10 FDD | R.13 FDD | R.6 FDD | R.7 FDD | R.9 FDD |
| Channel bandwidth | MHz | 1.4 | 5 | 5 | 20 | 20 | 10 | 10 | 10 |
| Number of transmitter antennas |  | 1 | 1 | 2 | 1 | 2 | 1 | 2 | 2 |
| Control region OFDM symbolsNote1 | symbols | 4 | 3 | 3 | 2 | 2 | 2 | 2 | 3 |
| Aggregation level | CCE | 2 (Note 6) | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
| DCI Format |  | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 |
| Cell ID |  | Note 4 | Note 4 | Note 4 | Note 4 | Note 4 | Note 4 | Note 4 | Note 4 |
| Payload (without CRC) | Bits | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 |
| NOTE 1: The control region consists of PCFICH, PHICH and PDCCH. NOTE 2: DCI formats are defined in 3GPP TS 36.212. NOTE 3: DCI format shall depend upon the test configuration. NOTE 4: Cell ID shall depend upon the test configuration. NOTE 5: Payload size shall depend upon the test configuration. NOTE 6: For PDCCH using SI/RA/P-RNTI, Aggregation level 4 is used. |  |  |  |  |  |  |  |  |  |

## A.2.2	TDD

Table A.2.2-1: PCFICH/PDCCH/PHICH Reference Channel for TDD

| Parameter | Unit | Value |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.8 TDD | R11 TDD | R.12 FDD | R.10 TDD | R.13 TDD | R.6 TDD | R.7 TDD | R.9 TDD |
| Channel bandwidth | MHz | 1.4 | 5 | 5 | 20 | 20 | 10 | 10 | 10 |
| Number of transmitter antennas |  | 1 | 1 | 2 | 1 | 2 | 1 | 2 | 2 |
| Control region OFDM symbolsNote1 | symbols | 4 (Note 6) | 3 | 3 | 2 | 2 | 2 | 2 | 3 |
| Aggregation level | CCE | 2 (Note 7) | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
| DCI Format |  | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 |
| Cell ID |  | Note 4 | Note 4 | Note 4 | Note 4 | Note 4 | Note 4 | Note 4 | Note 4 |
| Payload (without CRC) | Bits | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 |
| NOTE 1: The control region consists of PCFICH, PHICH and PDCCH. NOTE 2: DCI formats are defined in 3GPP TS 36.212. NOTE 3: DCI format shall depend upon the test configuration. NOTE 4: Cell ID shall depend upon the test configuration. NOTE 5: Payload size shall depend upon the test configuration. NOTE 6: Only 2 OFDM symbols for special subframes 1 and 6. NOTE 7: For PDCCH using SI/RA/P-RNTI, Aggregation level 4 is used. |  |  |  |  |  |  |  |  |  |

## A.2.3	HD-FDD for UE category 0

Table A.2.3-1: PCFICH/PDCCH/PHICH Reference Channel for HD-FDD

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | R.3 HD-FDD | R.4 HD-FDD | R.5 HD-FDD |
| Channel bandwidth | MHz | 10 | 10 | 10 |
| Number of transmitter antennas |  | 1 | 2 | 2 |
| Control region OFDM symbolsNote1 | symbols | 2 | 2 | 3 |
| Aggregation level | CCE | 8 | 8 | 8 |
| DCI Format |  | Note 3 | Note 3 | Note 3 |
| Cell ID |  | Note 4 | Note 4 | Note 4 |
| Payload (without CRC) | Bits | Note 5 | Note 5 | Note 5 |
| NOTE 1: The control region consists of PCFICH, PHICH and PDCCH. NOTE 2: DCI formats are defined in 3GPP TS 36.212. NOTE 3: DCI format shall depend upon the test configuration. NOTE 4: Cell ID shall depend upon the test configuration. NOTE 5: Payload size shall depend upon the test configuration. NOTE 6: For PDCCH using SI/RA/P-RNTI, Aggregation level 4 is used. NOTE 7: Sub-frame 0 or sub-frame 5 is randomly scheduled in downlink. |  |  |  |  |

## A.2.4	FS 3

Table A.2.4-1: PCFICH/PDCCH/PHICH Reference Channel for FS 3

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  |  |  |  | R.0 FS3 |  |
| Channel bandwidth | MHz | 10 | 10 | 10 | 20 |  |
| Number of transmitter antennas |  |  |  |  | 1 |  |
| Control region OFDM symbolsNote1 | symbols |  |  |  | 2 |  |
| Aggregation level | CCE |  |  |  | 8 |  |
| DCI Format |  |  |  |  | Note 3 |  |
| Cell ID |  |  |  |  | Note 4 |  |
| Payload (without CRC) | Bits |  |  |  | Note 5 |  |
| NOTE 1: The control region consists of PCFICH, PHICH and PDCCH. NOTE 2: DCI formats are defined in 3GPP TS 36.212. NOTE 3: DCI format shall depend upon the test configuration. NOTE 4: Cell ID shall depend upon the test configuration. NOTE 5: Payload size shall depend upon the test configuration. NOTE 6: For PDCCH using SI/RA/P-RNTI, Aggregation level 4 is used. NOTE 7: PCFICH/PDCCH/PHICH allocation applies only to subframes where there is no DRS transmission NOTE 8: PCFICH/PDCCH/PHICH are not transmitted in subframes where it is determined that transmission should not occur according to the listen before talk (LBT model). |  |  |  |  |  |  |

# A.3	PUSCH

This rule applies to E-UTRA cell(s), which the UE is connected to. The UE is in RRC-CONNECTED mode.

When signalling or data payloads are expected to be sent on the PUSCH, the UE may be provided in advance with PUSCH resources by the SS. For sake of simplicity the PUSCH scheduling may also occur continuously over many consecutive subframes. These options shall not be used if:

1)	stated otherwise in the test description; or

2)	the transmission of PUSCH and UL scheduling information affects the test purpose (e.g. DRX, PUCCH reception etc.)

For handover test cases, after RRC Connection reconfiguration message implying handover is sent, the UE shall be provided continuously with PUSCH resources by the SS in the source cell. This is done in order to make the requirement UE implementation agnostic, w.r.t. different delays caused by different handling of positive RLC acknowledgements, which are not mandatory and of lower priority than the handover procedure progress (clause 5.3.5.4 [5]).

If a PUSCH scheduling occurs, the SS sends uplink scheduling information via PDCCH DCI format 0 for C-RNTI to the UE. The UE sends uplink MAC padding bits on the PUSCH.

# A.4	Reference Measurement Channels for ProSe Direct Discovery

## A.4.1	FDD

Table A.4.1-1: PSDCH Reference Measurement Channels for FDD

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | D.1 FDD |
| Channel bandwidth | MHz | 5 |
| Allocated resource blocks |  | 2 |
| Subcarriers per resource block |  | 12 |
| Allocated subframes per Discovery period |  | 1 |
| DFT-OFDM Symbols per subframe (see note) |  | 11 |
| Modulation |  | QPSK |
| Information Bit Payload |  | 232 |
| Transport block CRC | Bits | 24 |
| Maximum number of HARQ transmissions |  | 1 |
| Binary Channel Bits (see note) | Bits | 528 |
| Note: PSDCH transmissions are rate-matched for 12 DFT-OFDM symbols per subframe, and the last symbol shall be punctured as per 3GPP TS 36.211. |  |  |

# A.5	Reference measurement channels for ProSe Direct Communication

## A.5.1	FDD

Table A.5.1-1: PSCCH Reference Measurement Channels for FDD

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
| Reference channel |  |  | CC.1 FDD | CC.2 FDD |
| Channel bandwidth |  | MHz | 5 | 10 |
| Allocated resource blocks |  |  | 1 | 1 |
| Subcarriers per resource block |  |  | 12 | 12 |
| DFT-OFDM Symbols per subframe (see Note 1) |  |  | 11 | 11 |
| Modulation |  |  | QPSK | QPSK |
| Information Bit Payload |  | Bits | 41 | 43 |
| Information bits | Frequency hopping flag |  | 0 |  |
|  | RB assignment |  | Set as per PSSCH RB allocation specific in the test |  |
|  | Time resource pattern (ITRP) |  | 0 (Note 2) |  |
|  | Modulation and coding scheme |  | Set as the PSSCH MCS specified in the test |  |
|  | Timing advance indication |  | 0 |  |
|  | Group destination ID |  | As set by higher layers |  |
| Transport block CRC |  | Bits | 16 | 16 |
| Maximum number of HARQ transmissions |  |  | 2 | 2 |
| Binary Channel Bits (see Note 1) |  | Bits | 264 | 264 |
| Note 1: PSCCH transmissions are rate-matched for 12 DFT-OFDM symbols per subframe, and the last symbol shall be punctured as per 3GPP TS 36.211. NOTE 2: For NTRP = 8 (FDD) and trpt-Subset = 001, ITRP = 0 corresponds to a time repetition pattern of (1,0,0,0,0,0,0,0) as per 3GPP TS 36.213. |  |  |  |  |

Table A.5.1-2: PSSCH Reference Measurement Channels for FDD

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | CD.1 FDD | CD.2 FDD |
| Channel bandwidth | MHz | 5 | 10 |
| Allocated resource blocks |  | 2 | 3 |
| Subcarriers per resource block |  | 12 | 12 |
| DFT-OFDM Symbols per subframe (see Note) |  | 11 | 11 |
| Modulation |  | QPSK | QPSK |
| Target Code Rate |  | 1/3 | 1/3 |
| Information Bit Payload |  | 176 | 256 |
| Transport block CRC | Bits | 24 | 24 |
| Maximum number of HARQ transmissions |  | 3 | 3 |
| Binary Channel Bits (see note) | Bits | 528 | 1056 |
| Note: PSDCH transmissions are rate-matched for 12 DFT-OFDM symbols per subframe, and the last symbol shall be punctured as per 3GPP TS 36.211. |  |  |  |

# A.6	ProSe Receive Traffic Generator

This clause defines the configuration for active Sidelink UEs used to generate receive traffic in ProSe RRM tests.

## A.6.1	ProSe Direct Communication Receive Traffic Generator for FDD

Table A.6.1-1: Active Sidelink UE configuration for ProSe Direct Communication

| Configuration |  |  | PCP.1.FDD |
| --- | --- | --- | --- |
| Channel BW |  | MHz | 5 or 10 |
| Number of Active Sidelink UEs per sc-period |  |  | 5 MHz: 12 10 MHz: 16 |
| Active Sidelink UEs | PSCCH RMC (defined in A.5) |  | 5 MHz: CC.1 FDD 10 MHz: CC.2 FDD |
|  | PSCCH resource allocation |  | 5MHz: [2i:2i], for Sidelink UE i=0,..,11 10MHz:[3i:3i], for Sidelink UE i = 0, .., 15 |
|  | PSSCH RMC (defined in A.5) |  | 5 MHz: CD.1 FDD 10 MHz: CD.2 FDD |
|  | PSSCH resource allocation |  | Non-overlapping RBs 5MHz: [2i:2i+1], for Sidelink UE i = 0, .., 11 10MHz:[3i:3i+2], for Sidelink UE i = 0, .., 15 |
|  | RSRP | dBm/15kHz | -98 |

## A.6.2	ProSe Direct Discovery Receive Traffic Generator for FDD

Table A.6.2-1: Active Sidelink UE configuration for ProSe Direct Discovery

| Configuration |  |  | PDP.1.FDD | PDP.2.FDD |
| --- | --- | --- | --- | --- |
| Channel BW |  | MHz | 5 |  |
| Number of Active Sidelink UEs per Discovery subframe |  |  | 12 |  |
| Active Sidelink UEs | Sidelink UE Transmissions |  | PSDCH (RMC D.1 FDD) | PSDCH (RMC D.1 FDD) + SLSS on synchronization subframe |
|  | Resource allocation |  | Non overlapping RBs in a subframe |  |
|  | RSRP | dBm/15kHz | -95 |  |

# A.7	MPDCCH Reference Channels for Cat-M1 UEs

MPDCCH reference measurement channels in this clause can be used in tests for Cat-M2 UEs.

## A.7.1	FDD in CEModeA

Table A.7.1-1: MPDCCH Reference Channel for Cat-M1 FDD UEs in CEModeA

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| MPDCCH Reference channel | - | R.16 FDD | R.17 FDD | R.24 FDD | R.25 FDD |
| Carrier bandwidth | MHz | 10 | 10 | 5 | 5 |
| Number of transmitter antennas | - | 1 | 2 | 1 | 2 |
| DCI Format | - | 6-1A | 6-1A | 6-1A | 6-1A |
| Transmission Type | - | Distributed | Distributed | Distributed | Distributed |
| Number of PRB pairs per M-PDCCH set | - | 4 | 4 | 4 | 4 |
| Aggregation level | ECCE | 16 | 16 | 16 | 16 |
| Maximum number of repetitions | - | 8 | 8 | 8 | 8 |
| Frequency hopping | - | ON | ON | ON | ON |
| Number of narrowbands | - | 2 | 2 | 2 | 2 |
| MPDCCH Narrowband | - | 7th | 7th | 4th | 4th |
| Frequency HoppingOffset | narrowbands | 7 | 7 | 3 | 3 |
| MPDCCH start subframe | subframes | 1 | 1 | 1 | 1 |
| MPDCCH start symbol | symbols | 2 | 2 | 2 | 2 |
| Frequency hopping interval | subframes | 4 | 4 | 4 | 4 |
| Payload (without CRC) | Bits | Note 1 | Note 1 | Note 1 | Note 1 |
| Cell ID | - | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Payload size shall depend upon the test configuration. NOTE 2: Cell ID shall depend upon the test configuration. |  |  |  |  |  |

## A.7.2	HD-FDD in CEModeA

Table A.7.2-1: MPDCCH Reference Channel for Cat-M1 HD-FDD UEs in CEModeA

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| MPDCCH Reference channel | - | R.6 HD-FDD | R.7 HD-FDD | R.14 HD-FDD | R.15 HD-FDD |
| Carrier bandwidth | MHz | 10 | 10 | 5 | 5 |
| Number of transmitter antennas | - | 1 | 2 | 1 | 2 |
| DCI Format | - | 6-1A | 6-1A | 6-1A | 6-1A |
| Transmission Type | - | Distributed | Distributed | Distributed | Distributed |
| Number of PRB pairs per M-PDCCH set | - | 4 | 4 | 4 | 4 |
| Aggregation level | ECCE | 16 | 16 | 16 | 16 |
| Maximum number of repetitions | - | 8 | 8 | 8 | 8 |
| Frequency hopping | - | ON | ON | ON | ON |
| Number of narrowbands | - | 2 | 2 | 2 | 2 |
| MPDCCH Narrowband | - | 7th | 7th | 4th | 4th |
| Frequency HoppingOffset | narrowbands | 7 | 7 | 3 | 3 |
| MPDCCH start subframe | subframes | 1 | 1 | 1 | 1 |
| MPDCCH start symbol | symbols | 2 | 2 | 2 | 2 |
| Frequency hopping interval | subframes | 4 | 4 | 4 | 4 |
| Payload (without CRC) | Bits | Note 1 | Note 1 | Note 1 | Note 1 |
| Cell ID | - | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Payload size shall depend upon the test configuration. NOTE 2: Cell ID shall depend upon the test configuration. |  |  |  |  |  |

## A.7.3	TDD in CEModeA

Table A.7.3-1: MPDCCH Reference Channel for Cat-M1 TDD UEs in CEModeA

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| MPDCCH Reference channel | - | R.14 TDD | R.15 TDD |
| Carrier bandwidth | MHz | 10 | 10 |
| Number of transmitter antennas | - | 1 | 2 |
| DCI Format | - | 6-1A | 6-1A |
| Transmission Type | - | Distributed | Distributed |
| Number of PRB pairs per M-PDCCH set | - | 4 | 4 |
| Aggregation level | ECCE | 16 | 16 |
| Maximum number of repetitions | - | 8 | 8 |
| Frequency hopping | - | ON | ON |
| Number of narrowbands | - | 2 | 2 |
| MPDCCH Narrowband | - | 7th | 7th |
| Frequency HoppingOffset | narrowbands | 7 | 7 |
| MPDCCH start subframe | subframes | 1 | 1 |
| MPDCCH start symbol | symbols | 2 | 2 |
| Frequency hopping interval | subframes | 10 | 10 |
| Payload (without CRC) | Bits | Note 1 | Note 1 |
| Cell ID | - | Note 2 | Note 2 |
| NOTE 1: Payload size shall depend upon the test configuration. NOTE 2: Cell ID shall depend upon the test configuration. |  |  |  |

## A.7.4	FDD in CEModeB

Table A.7.4-1: MPDCCH Reference Channel for Cat-M1 FDD UEs in CEModeB

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| MPDCCH Reference channel | - | R.18 FDD | R.19 FDD | R.26 FDD | R.27 FDD |
| Carrier bandwidth | MHz | 10 | 10 | 5 | 5 |
| Number of transmitter antennas | - | 1 | 2 | 1 | 2 |
| DCI Format | - | 6-1B | 6-1B | 6-1B | 6-1B |
| Transmission Type | - | Distributed | Distributed | Distributed | Distributed |
| Number of PRB pairs per M-PDCCH set | - | 6 | 6 | 6 | 6 |
| Aggregation level | ECCE | 24 | 24 | 24 | 24 |
| Maximum number of repetitions | - | 128 | 128 | 128 | 128 |
| Frequency hopping | - | ON | ON | ON | ON |
| Number of narrowbands | - | 2 | 2 | 2 | 2 |
| MPDCCH Narrowband | - | 7th | 7th | 4th | 4th |
| Frequency HoppingOffset | narrowbands | 7 | 7 | 3 | 3 |
| MPDCCH start subframe | subframes | 1 | 1 | 1 | 1 |
| MPDCCH start symbol | symbols | 2 | 2 | 2 | 2 |
| Frequency hopping interval | subframes | 8 | 8 | 8 | 8 |
| Payload (without CRC) | Bits | 17 | 17 | 17 | 17 |
| Cell ID | - | Note | Note | Note | Note |
| NOTE: Cell ID shall depend upon the test configuration. |  |  |  |  |  |

## A.7.5	HD-FDD in CEModeB

Table A.7.5-1: MPDCCH Reference Channel for Cat-M1 HD-FDD UEs in CEModeB

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| MPDCCH Reference channel | - | R.8 HD-FDD | R.9 HD-FDD | R.16 HD-FDD | R.17 HD-FDD |
| Carrier bandwidth | MHz | 10 | 10 | 5 | 5 |
| Number of transmitter antennas | - | 1 | 2 | 1 | 2 |
| DCI Format | - | 6-1B | 6-1B | 6-1B | 6-1B |
| Transmission Type | - | Distributed | Distributed | Distributed | Distributed |
| Number of PRB pairs per M-PDCCH set | - | 6 | 6 | 6 | 6 |
| Aggregation level | ECCE | 24 | 24 | 24 | 24 |
| Maximum number of repetitions | - | 128 | 128 | 128 | 128 |
| Frequency hopping | - | ON | ON | ON | ON |
| Number of narrowbands | - | 2 | 2 | 2 | 2 |
| MPDCCH Narrowband | - | 7th | 7th | 4th | 4th |
| Frequency HoppingOffset | narrowbands | 7 | 7 | 3 | 3 |
| MPDCCH start subframe | subframes | 1 | 1 | 1 | 1 |
| MPDCCH start symbol | symbols | 2 | 2 | 2 | 2 |
| Frequency hopping interval | subframes | 8 | 8 | 8 | 8 |
| Payload (without CRC) | Bits | 17 | 17 | 17 | 17 |
| Cell ID | - | Note | Note | Note | Note |
| NOTE: Cell ID shall depend upon the test configuration. |  |  |  |  |  |

## A.7.6	TDD in CEModeB

Table A.7.6-1: MPDCCH Reference Channel for Cat-M1 TDD UEs in CEModeB

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| MPDCCH Reference channel | - | R.16 TDD | R.17 TDD |
| Carrier bandwidth | MHz | 10 | 10 |
| Number of transmitter antennas | - | 1 | 2 |
| DCI Format | - | 6-1B | 6-1B |
| Transmission Type | - | Distributed | Distributed |
| Number of PRB pairs per M-PDCCH set | - | 6 | 6 |
| Aggregation level | ECCE | 24 | 24 |
| Maximum number of repetitions | - | 128 | 128 |
| Frequency hopping | - | ON | ON |
| Number of narrowbands | - | 2 | 2 |
| MPDCCH Narrowband | - | 7th | 7th |
| Frequency HoppingOffset | narrowbands | 7 | 7 |
| MPDCCH start subframe | subframes | 1 | 1 |
| MPDCCH start symbol | symbols | 2 | 2 |
| Frequency hopping interval | subframes | 20 | 20 |
| Payload (without CRC) | Bits | 17 | 17 |
| Cell ID | - | Note | Note |
| NOTE: Cell ID shall depend upon the test configuration. |  |  |  |

# A.8	PDSCH Reference Channels for Cat-M1 UEs

## A.8.1	FDD in CEModeA

Table A.8.1-1: PDSCH Reference Channel for Cat-M1 FDD in CEModeA

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PDSCH Reference channel |  | R.20 FDD | R.21 FDD | R.28 FDD | R.29 FDD | R.32 FDD | R.33 FDD |
| Carrier bandwidth | MHz | 10 | 10 | 5 | 5 | 10 | 10 |
| Number of transmitter antennas |  | 1 | 2 | 1 | 2 | 1 | 2 |
| Allocated resource blocksNote1 | PRBs | 2 | 2 | 2 | 2 | 2 | 2 |
| Allocated subframes per Radio Frame | subframes | 10 | 10 | 10 | 10 | 10 | 10 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |  |  |  |  |
| Sub-Frames 0 ~ 9 | Bits | 32 | 32 | 32 | 32 | 32 | 32 |
| Number of Code Blocks per Sub-Frame |  |  |  |  |  |  |  |
| Sub-Frames 0 ~ 9 |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Maximum number of repetitions |  | 16 | 16 | 16 | 16 | 1 | 1 |
| Frequency hopping |  | ON | ON | ON | ON | ON | ON |
| Number of narrowbands for frequency hopping | narrowbands | 2 | 2 | 2 | 2 | 2 | 2 |
| PDSCH Narrowband |  | 2nd | 2nd | 2nd | 2nd | 2nd | 2nd |
| Frequency HoppingOffset | narrowbands | 7 | 7 | 3 | 3 | 7 | 7 |
| PDSCH start symbol | symbols | 2 | 2 | 2 | 2 | 2 | 2 |
| Frequency hopping interval | subframes | 4 | 4 | 4 | 4 | 4 | 4 |
| Max. Throughput averaged over 1 frame | kbps | TBD | TBD | TBD | TBD | TBD | TBD |
| Cell ID |  | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Allocation is located in the middle of narrowband. NOTE 2: Cell ID shall depend upon the test configuration. |  |  |  |  |  |  |  |

## A.8.2	HD-FDD in CEModeA

Table A.8.2-1: PDSCH Reference Channel for Cat-M1 HD-FDD in CEModeA

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PDSCH Reference channel |  | R.10 HD-FDD | R.11 HD-FDD | R.18 HD-FDD | R.19 HD-FDD | R.24 HD-FDD | R.25 HD-FDD |
| Carrier bandwidth | MHz | 10 | 10 | 5 | 5 | 10 | 10 |
| Number of transmitter antennas |  | 1 | 2 | 1 | 2 | 1 | 2 |
| Allocated resource blocksNote1 | PRBs | 2 | 2 | 2 | 2 | 2 | 2 |
| Allocated subframes per Radio Frame | subframes | 10 | 10 | 10 | 10 | 10 | 10 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |  |  |  |  |
| Sub-Frames 0 ~ 9 | Bits | 32 | 32 | 32 | 32 | 32 | 32 |
| Number of Code Blocks per Sub-Frame |  |  |  |  |  |  |  |
| Sub-Frames 0 ~ 9 |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Maximum number of repetitions |  | 16 | 16 | 16 | 16 | 1 | 1 |
| Frequency hopping |  | ON | ON | ON | ON | ON | ON |
| Number of narrowbands for frequency hopping | narrowbands | 2 | 2 | 2 | 2 | 2 | 2 |
| PDSCH Narrowband |  | 1st | 1st | 1st | 1st | 1st | 1st |
| Frequency HoppingOffset | narrowbands | 7 | 7 | 3 | 3 | 7 | 7 |
| PDSCH start symbol | symbols | 2 | 2 | 2 | 2 | 2 | 2 |
| Frequency hopping interval | subframes | 4 | 4 | 4 | 4 | 4 | 4 |
| Max. Throughput averaged over 1 frame | kbps | TBD | TBD | TBD | TBD | TBD | TBD |
| Cell ID |  | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Allocation is located in the middle of narrowband. NOTE 2: Cell ID shall depend upon the test configuration. |  |  |  |  |  |  |  |

## A.8.3	TDD in CEModeA

Table A.8.3-1: PDSCH Reference Channel for Cat-M1 TDD in CEModeA

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| PDSCH Reference channel |  | R.16 TDD | R.17 TDD |
| Carrier bandwidth | MHz | 10 | 10 |
| Number of transmitter antennas |  | 1 | 2 |
| Allocated resource blocksNote1 | PRBs | 2 | 2 |
| Allocated subframes per Radio Frame |  | 4 | 4 |
| TDD Uplink-Downlink Configuration |  | 0 | 0 |
| TDD Special Subframe Configuration |  | 6 | 6 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |
| All Sub-Frames except 1, 6 (TDD) | Bits | 32 | 32 |
| All Sub-Frames except 1, 6 (TDD DwPTS) | - | 0 | 0 |
| Number of Code Blocks per Sub-Frame |  | 1 | 1 |
| All Sub-Frames except 1, 6 (TDD) |  | 1 | 1 |
| All Sub-Frames except 1, 6 (TDD DwPTS) |  | 0 | 0 |
| Maximum number of repetitions |  | 16 | 16 |
| Frequency hopping |  | ON | ON |
| Number of narrowbands for frequency hopping |  | 2 | 2 |
| PDSCH Narrowband |  | 2nd | 2nd |
| Frequency HoppingOffset | narrowbands | 7 | 7 |
| PDSCH start symbol | symbols | 2 | 2 |
| Frequency hopping interval | subframes | 10 | 10 |
| Max. Throughput averaged over 1 frame | kbps | TBD | TBD |
| Cell ID |  | Note 2 | Note 2 |
| NOTE 1: Allocation is located in the middle of narrowband. NOTE 2: Cell ID shall depend upon the test configuration. |  |  |  |

## A.8.4	FDD in CEModeB

Table A.8.4-1: PDSCH Reference Channel for Cat-M1 FDD in CEModeB

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PDSCH Reference channel |  | R.22 FDD | R.23 FDD | R.30 FDD | R.31 FDD |
| Carrier bandwidth | MHz | 10 | 10 | 5 | 5 |
| Number of transmitter antennas |  | 1 | 2 | 1 | 2 |
| Allocated resource blocksNote1 |  | 2 | 2 | 2 | 2 |
| Allocated subframes per Radio Frame | - | 10 | 10 | 10 | 10 |
| Modulation | - | QPSK | QPSK | QPSK | QPSK |
| Target coding rate | - | 1/3 | 1/3 | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |  |  |
| Sub-Frames 0 ~ 9 | Bits | 32 | 32 | 32 | 32 |
| Number of Code Blocks per Sub-Frame |  |  |  |  |  |
| Sub-Frames 0 ~ 9 | - | 1 | 1 | 1 | 1 |
| Maximum number of repetitions | - | 192 | 192 | 192 | 192 |
| Frequency hopping | - | ON | ON | ON | ON |
| Number of narrowbands for frequency hopping | narrowbands | 2 | 2 | 2 | 2 |
| PDSCH Narrowband | - | 2nd | 2nd | 2nd | 2nd |
| Frequency HoppingOffset | narrowbands | 7 | 7 | 3 | 3 |
| PDSCH start symbol | symbols | 2 | 2 | 2 | 2 |
| Frequency hopping interval | subframes | 8 | 8 | 8 | 8 |
| Max. Throughput averaged over 1 frame | kbps | TBD | TBD | TBD | TBD |
| Cell ID |  | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Allocation is located in the middle of narrowband. NOTE 2: Cell ID shall depend upon the test configuration. |  |  |  |  |  |

## A.8.5	HD-FDD in CEModeB

Table A.8.5-1: PDSCH Reference Channel for Cat-M1 HD-FDD in CEModeB

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PDSCH Reference channel |  | R.12 HD-FDD | R.13 HD-FDD | R.20 HD-FDD | R.21 HD-FDD |
| Carrier bandwidth | MHz | 10 | 10 | 5 | 5 |
| Number of transmitter antennas |  | 1 | 2 | 1 | 2 |
| Allocated resource blocksNote1 | PRBs | 2 | 2 | 2 | 2 |
| Allocated subframes per Radio Frame | subframes | 10 | 10 | 10 | 10 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |  |  |
| Sub-Frames 0 ~ 9 | Bits | 32 | 32 | 32 | 32 |
| Number of Code Blocks per Sub-Frame |  |  |  |  |  |
| Sub-Frames 0 ~ 9 |  | 1 | 1 | 1 | 1 |
| Maximum number of repetitions |  | 192 | 192 | 192 | 192 |
| Frequency hopping |  | ON | ON | ON | ON |
| Number of narrowbands for frequency hopping | narrowbands | 2 | 2 | 2 | 2 |
| PDSCH Narrowband |  | 1st | 1st | 1st | 1st |
| Frequency HoppingOffset | narrowbands | 7 | 7 | 3 | 3 |
| PDSCH start symbol | symbols | 2 | 2 | 2 | 2 |
| Frequency hopping interval | subframes | 8 | 8 | 8 | 8 |
| Max. Throughput averaged over 1 frame | kbps | TBD | TBD | TBD | TBD |
| Cell ID |  | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Allocation is located in the middle of narrowband. NOTE 2: Cell ID shall depend upon the test configuration. |  |  |  |  |  |

## A.8.6	TDD in CEModeB

Table A.8.6-1: PDSCH Reference Channel for Cat-M1 TDD in CEModeB

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| PDSCH Reference channel |  | R.18 TDD | R.19 TDD |
| Carrier bandwidth | MHz | 10 | 10 |
| Number of transmitter antennas |  | 1 | 2 |
| Allocated resource blocksNote1 | PRBs | 2 | 2 |
| Allocated subframes per Radio Frame |  | 4 | 4 |
| TDD Uplink-Downlink Configuration |  | 0 | 0 |
| TDD Special Subframe Configuration |  | 6 | 6 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |
| All Sub-Frames except 1, 6 (TDD) | Bits | 32 | 32 |
| All Sub-Frames except 1, 6 (TDD DwPTS) | - | 0 | 0 |
| Number of Code Blocks per Sub-Frame |  | 1 | 1 |
| All Sub-Frames except 1, 6 (TDD) |  | 1 | 1 |
| All Sub-Frames except 1, 6 (TDD DwPTS) |  | 0 | 0 |
| Maximum number of repetitions |  | 192 | 192 |
| Frequency hopping |  | ON | ON |
| Number of narrowbands for frequency hopping |  | 2 | 2 |
| PDSCH Narrowband |  | 2nd | 2nd |
| Frequency HoppingOffset | narrowbands | 7 | 7 |
| PDSCH start symbol | symbols | 2 | 2 |
| Frequency hopping interval | subframes | 20 | 20 |
| Max. Throughput averaged over 1 frame | kbps | TBD | TBD |
| Cell ID |  | Note 2 | Note 2 |
| NOTE 1: Allocation is located in the middle of narrowband. NOTE 2: Cell ID shall depend upon the test configuration. |  |  |  |

# A.8a	Reference Configurations for satellites for Cat-M1 UEs

## A.8a.1	General setup for SIB31

The general parameters for SIB31 setup is specified in Table A.8a.1-1.

Table A.8a.1-1: General setup for SIB31

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| Interval between adjacent epoch time | s | LEO: 2.56 GEO, GSO: 10.24 |
| ntn-UlSyncValidityDuration | s | LEO: 5s GEO, GSO: 900s |
| k-Offset-r17 | ms | LEO: 14 GEO, GSO: 258 |
| kmac | ms | Not configured |
| ta-Common |  | 0 |
| ta-CommonDrift |  | 0 |
| ta-CommonDriftVariant |  | 0 |
| ephemerisInfo |  | According to TS 36.133 [5] Annex B.8 |
| ta-Report |  | Not configured |

## A.8a.2	Satellite specific configuration for serving cell

The general parameters for SIB31 setup for serving satellite are specified in Table A.8a.2-1.

Table A.8a.2-1: SIB31 parameters setup for Serving satellite

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference configuration for serving satellite |  | SSC.1 | SSC.2 |
| Scenario |  | GSO or GEO (as per test definition) | NGSO |
| Interval between adjacent epoch time | s | 10.24 | 2.56 |
| ul-SyncValidityDuration-r17 | s | 900 | 5 |
| k-Offset-r17 | ms | 258 | 14 |
| k-Mac-r17 | ms | Not configured | Not configured |
| nta-Common-r17 |  | 0 | 0 |
| nta-CommonDrift-r17 |  | 0 | 0 |
| nta-CommonDriftVariation-r17 |  | 0 | 0 |
| ephemerisInfo |  | According to TS 36.133 [5] Annex B.8 |  |

## A.8a.3	Satellite specific configuration for neighbour cell

The general parameters for SIB33 setup for neighbour cells in both serving and neighbour satellites are specified in Table A.8a.3-1.

Table A.8a.3-1: SIB33 parameters setup for neighbour cell

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference configuration for satellite assistance information for neighbour cell |  | NSC.1 | NSC.2 | NSC.3 | NSC.4 |
| Scenario |  | GSO or GEO (as per test definition) | NGSO | GSO or GEO (as per test definition) | NGSO |
| Interval between adjacent epoch time | s | 10.24 | 2.56 | 10.24 | 2.56 |
| neighValidityDuration-r18 | s | 900 | 5 | 900 | 5 |
| k-Mac-r18 | ms | Not configured | Not configured | Not configured | Not configured |
| nta-Common-r18 |  | 0 | 0 | 0 | 0 |
| nta-CommonDrift-r18 |  | 0 | 0 | 0 | 0 |
| nta-CommonDriftVariation-r18 |  | 0 | 0 | 0 | 0 |
| ephemerisInfo |  | Same as used for serving satellite |  | According to TS 36.133 [5] Annex B.8 NOTE1 |  |
| t-ServiceStartNeigh-r18 |  | NOTE 2 |  |  |  |
| Note 1: The ephemeris information for those neighbour cells shall be different than that used for the serving cell, i.e., these cells are associated to a different satellite. Note 2: Unless otherwise stated in the test case, t-serviceStartNeigh-r18 is not configured for the neighbour satellite. |  |  |  |  |  |

# A.9	Reference PRACH Configurations

Table A.9-1: PRACH configuration parameters

| Parameter | Value |  |  | Comments |
| --- | --- | --- | --- | --- |
| Reference configuration | PRACH_2CE | PRACH_3CE | PRACH_4CE |  |
| Rsrp-ThresholdsPRACH | {-99} dBm | {-107, -99} dBm | {-107, -99, -92} dBm | As defined in 3GPP TS 36.331 [5] |
| CE level0: | Configured | Configured | Configured | Up to 4 CE levels, each corresponding to a PRACH configuration |
| Prach Configuration Index: | FDD：4 TDD：53 | FDD：4 TDD：53 | FDD：4 TDD：53 | See 3GPP TS 36.211 [9] clause 5.7.1 |
| numRepetitionPerPreambleAttempt: | 1 | 1 | 1 | Number of PRACH repetitions per attempt for each CE level, See 3GPP TS 36.211 [9] |
| prach-HoppingConfig: | off | off | off | Coverage level specific frequency hopping configuration for PRACH |
| CE level1: | Configured | Configured | Configured | Up to 4 CE levels, each corresponding to a PRACH configuration |
| Prach Configuration Index: | FDD：4 TDD：53 | FDD：4 TDD：53 | FDD：4 TDD：53 | see 3GPP TS 36.211 [9] clause 5.7.1 |
| numRepetitionPerPreambleAttempt: | 128 | 64 | 32 | Number of PRACH repetitions per attempt for each CE level, See 3GPP TS 36.211 [9] |
| prach-HoppingConfig: | off | off | off | Coverage level specific frequency hopping configuration for PRACH |
| CE level2: | Not Configured | Configured | Configured | Up to 4 CE levels, each corresponding to a PRACH configuration |
| Prach Configuration Index: | - | FDD：4 TDD：53 | FDD：4 TDD：53 | see 3GPP TS 36.211 [9] clause 5.7.1 |
| numRepetitionPerPreambleAttempt: | - | 128 | 64 | Number of PRACH repetitions per attempt for each CE level, See 3GPP TS 36.211 [9] |
| prach-HoppingConfig: | - | off | off | Coverage level specific frequency hopping configuration for PRACH |
| CE level3: | Not Configured | Not Configured | Configured | Up to 4 CE levels, each corresponding to a PRACH configuration |
| Prach Configuration Index: | - | - | FDD：4 TDD：53 | see 3GPP TS 36.211 [9] clause 5.7.1 |
| numRepetitionPerPreambleAttempt: | - | - | 128 | Number of PRACH repetitions per attempt for each CE level, See 3GPP TS 36.211 [9] |
| prach-HoppingConfig: | - | - | off | Coverage level specific frequency hopping configuration for PRACH |

# A.10	General configuration for NB-IoT

This clause defines the configuration for category NB1 UEs used in NB-IoT RRM tests.

## A.10.1	NPDCCH Reference Channel for UE category NB1

### A.10.1.1	In-band operation

Table A.10.1.1-1: NPDCCH Reference Channel for
UE category NB1 for in-band operation in 10MHz LTE system

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| NPDCCH Reference channel | - | R.26 HD-FDD R.26 NB-TDD | R.27 HD-FDD R.27 NB-TDD | R.28 HD-FDD R.28 NB-TDD | R.29 HD-FDD R.29 NB-TDD |
| LTE Carrier bandwidth | MHz | 10 | 10 | 10 | 10 |
| Allocated resource blocks Note1 | PRB | 1 | 1 | 1 | 1 |
| Number of transmitter antennas | - | 1 | 2 | 1 | 2 |
| DCI Format | - | N1 | N1 | N0 | N0 |
| Aggregation level | NCCE | 2 | 2 | 2 | 2 |
| Maximum number of repetitions | - | Note 2 | Note 2 | Note 2 | Note 2 |
| NPDCCH start symbol | symbols | 3 | 3 | 3 | 3 |
| Payload (without CRC) | Bits | Note 3 | Note 3 | Note 3 | Note 3 |
| Cell ID | - | Note 4 | Note 4 | Note 4 | Note 4 |
| NOTE 1: Allocation is located in the middle of narrowband. NOTE 2: Maximum number of repetitions shall depend upon the test configuration. NOTE 3: Payload size shall depend upon the test configuration. NOTE 4: Cell ID shall depend upon the test configuration. |  |  |  |  |  |

Table A.10.1.1-2: NPDCCH Reference Channel for
UE category NB1 for in-band operation in 5MHz LTE system

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| NPDCCH Reference channel | - | R.38 HD-FDD | R.39 HD-FDD | R.40 HD-FDD | R.41 HD-FDD |
| LTE Carrier bandwidth | MHz | 5 | 5 | 5 | 5 |
| Allocated resource blocks Note1 | PRB | 1 | 1 | 1 | 1 |
| Number of transmitter antennas | - | 1 | 2 | 1 | 2 |
| DCI Format | - | N1 | N1 | N0 | N0 |
| Aggregation level | NCCE | 2 | 2 | 2 | 2 |
| Maximum number of repetitions | - | Note 2 | Note 2 | Note 2 | Note 2 |
| NPDCCH start symbol | symbols | 3 | 3 | 3 | 3 |
| Payload (without CRC) | Bits | Note 3 | Note 3 | Note 3 | Note 3 |
| Cell ID | - | Note 4 | Note 4 | Note 4 | Note 4 |
| NOTE 1: Allocation is located in the middle of narrowband. NOTE 2: Maximum number of repetitions shall depend upon the test configuration. NOTE 3: Payload size shall depend upon the test configuration. NOTE 4: Cell ID shall depend upon the test configuration. |  |  |  |  |  |

### A.10.1.2	Standalone operation

Table A.10.1.2-1: NPDCCH Reference Channel for UE category NB1 for standalone operation

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| NPDCCH Reference channel | - | R.30 HD-FDD R.30 NB-TDD | R.31 HD-FDD R.31 NB-TDD | R.32 HD-FDD R.32 NB-TDD | R.33 HD-FDD R.33 NB-TDD |
| Channel bandwidth | KHz | 200 | 200 | 200 | 200 |
| Number of transmitter antennas | - | 1 | 2 | 1 | 2 |
| DCI Format | - | N1 | N1 | N0 | N0 |
| Aggregation level | NCCE | 2 | 2 | 2 | 2 |
| Maximum number of repetitions | - | Note 2 | Note 2 | Note 2 | Note 2 |
| Payload (without CRC) | Bits | Note 3 | Note 3 | Note 3 | Note 3 |
| Cell ID | - | Note 4 | Note 4 | Note 4 | Note 4 |
| NOTE 1: Allocation is located in the middle of narrowband. NOTE 2: Maximum number of repetitions shall depend upon the test configuration. NOTE 3: Payload size shall depend upon the test configuration. NOTE 4: Cell ID shall depend upon the test configuration. |  |  |  |  |  |

### A.10.1.3	Guard band operation

Table A.10.1.3-1: NPDCCH Reference Channel for UE category NB1 for guard band operation

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| NPDCCH Reference channel | - | R.34 HD-FDD R.34 NB-TDD | R.35 HD-FDD R.35 NB-TDD | R.36 HD-FDD R.36 NB-TDD | R.37 HD-FDD R.37 NB-TDD |
| LTE Carrier bandwidth | MHz | 10 | 10 | 10 | 10 |
| Allocated resource blocks for NB-ioTNote1 | PRB | 1 | 1 | 1 | 1 |
| Number of transmitter antennas | - | 1 | 2 | 1 | 2 |
| DCI Format | - | N1 | N1 | N0 | N0 |
| Aggregation level | NCCE | 2 | 2 | 2 | 2 |
| Maximum number of repetitions | - | Note 2 | Note 2 | Note 2 | Note 2 |
| Payload (without CRC) | Bits | Note 3 | Note 3 | Note 3 | Note 3 |
| Cell ID | - | Note 4 | Note 4 | Note 4 | Note 4 |
| NOTE 1: Allocation is located in the middle of narrowband. NOTE 2: Maximum number of repetitions shall depend upon the test configuration. NOTE 3: Payload size shall depend upon the test configuration. NOTE 4: Cell ID shall depend upon the test configuration. |  |  |  |  |  |

## A.10.2	NPDSCH Reference Channel for UE category NB1

### A.10.2.1	HD-FDD in-band operation

Table A.10.2.1-1: NPDSCH Reference Channel for UE category NB1 for in-band operation

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| NPDSCH Reference channel | - | R.14 HD-FDD | R.15 HD-FDD | R.16 HD-FDD | R.17 HD-FDD |
| LTE Carrier bandwidth | MHz | 10 | 10 | 5 | 5 |
| Allocated resource blocks Note1 | PRB | 1 | 1 | 1 | 1 |
| Number of transmitter antennas | - | 1 | 2 | 1 | 2 |
| Allocated subframes per Radio Frame | subframes | Note 1 | Note 1 | Note 1 | Note 1 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |  |  |
| For Sub-Frame 1, 2, 3, 6, 7, 8 | Bits | 40 Note 2 | 40 Note 2 | 40 Note 2 | 40 Note 2 |
| For Sub-Frame 4, 9 | Bits | Note 3 | Note 3 | Note 3 | Note 3 |
| For Sub-Frame 0, 5 | Bits | 0 | 0 | 0 | 0 |
| Number of Code Blocks per Sub-Frame |  |  |  |  |  |
| For Sub-Frame 1, 2, 3, 6, 7, 8 |  | 1 Note 4 | 1 Note 4 | 1 Note 4 | 1 Note 4 |
| For Sub-Frame 4, 9 |  | Note 5 | Note 5 | Note 5 | Note 5 |
| For Sub-Frame 0, 5 |  | 0 | 0 | 0 | 0 |
| Maximum number of repetitions | - | Note 6 | Note 6 | Note 6 | Note 6 |
| NPDCCH start symbol | symbols | 3 | 3 | 3 | 3 |
| Cell ID | - | Note 7 | Note 7 | Note 7 | Note 7 |
| NOTE 1: Shall depend upon the NPDSCH scheduling. NOTE 2: Only apply for subframes scheduled with NPDSCH. NOTE 3: 40 for subframes scheduled with NPDSCH when mod 2 ≠ 0. Otherwise 0. NOTE 4: Only apply for subframes scheduled with NPDSCH. NOTE 5: 1 for subframes scheduled with NPDSCH when mod 2 ≠ 0. Otherwise 0. NOTE 6: Maximum number of repetitions shall depend upon the test configuration. NOTE 7: Cell ID shall depend upon the test configuration. |  |  |  |  |  |

### A.10.2.2	HD-FDD standalone operation

Table A.10.2.2-1: NPDSCH Reference Channel for UE category NB1 for standalone operation

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| NPDSCH Reference channel | - | R.18 HD-FDD | R.19 HD-FDD |
| Channel bandwidth | KHz | 200 | 200 |
| Number of transmitter antennas | - | 1 | 2 |
| Allocated subframes per Radio Frame | subframes | Note 1 | Note 1 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |
| For Sub-Frame 1, 2, 3, 6, 7, 8 | Bits | 72Note 2 | 72Note 2 |
| For Sub-Frame 4, 9 | Bits | Note 3 | Note 3 |
| For Sub-Frame 0, 5 | Bits | 0 | 0 |
| Number of Code Blocks per Sub-Frame |  |  |  |
| For Sub-Frame 1, 2, 3, 6, 7, 8 |  | 1Note 4 | 1Note 4 |
| For Sub-Frame 4, 9 |  | Note 5 | Note 5 |
| For Sub-Frame 0, 5 |  | 0 | 0 |
| Maximum number of repetitions | - | Note 6 | Note 6 |
| Cell ID | - | Note 7 | Note 7 |
| NOTE 1: Shall depend upon the NPDSCH scheduling. NOTE 2: only apply for subframes scheduled with NPDSCH NOTE 3: 72 for subframes scheduled with NPDSCH when mod 2 ≠ 0. Otherwise 0. NOTE 4: only apply for subframes scheduled with NPDSCH. NOTE 5: 1 for subframes scheduled with NPDSCH when mod 2 ≠ 0. Otherwise 0. NOTE 6: Maximum number of repetitions shall depend upon the test configuration. NOTE 7: Cell ID shall depend upon the test configuration. |  |  |  |

### A.10.2.3	HD-FDD guard band operation

Table A.10.2.3-1: NPDSCH Reference Channel for UE category NB1 for guard band operation

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| NPDSCH Reference channel | - | R.22 HD-FDD | R.23 HD-FDD |
| LTE Carrier bandwidth | MHz | 10 | 10 |
| Allocated resource blocks for NB-IoTNote1 | PRB | 1 | 1 |
| Number of transmitter antennas | - | 1 | 2 |
| Allocated subframes per Radio Frame | subframes | Note 1 | Note 1 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |
| For Sub-Frame 1, 2, 3, 6, 7, 8 | Bits | 72Note 2 | 72Note 2 |
| For Sub-Frame 4, 9 | Bits | Note 3 | Note 3 |
| For Sub-Frame 0, 5 | Bits | 0 | 0 |
| Number of Code Blocks per Sub-Frame |  |  |  |
| For Sub-Frame 1, 2, 3, 6, 7, 8 |  | 1Note 4 | 1Note 4 |
| For Sub-Frame 4, 9 |  | Note 5 | Note 5 |
| For Sub-Frame 0, 5 |  | 0 | 0 |
| Maximum number of repetitions | - | Note 6 | Note 6 |
| Cell ID | - | Note 7 | Note 7 |
| NOTE 1: Shall depend upon the NPDSCH scheduling. NOTE 2: only apply for subframes scheduled with NPDSCH. NOTE 3: 72 for subframes scheduled with NPDSCH when mod 2 ≠ 0. Otherwise 0. NOTE 4: only apply for subframes scheduled with NPDSCH. NOTE 5: 1 for subframes scheduled with NPDSCH when mod 2 ≠ 0. Otherwise 0. NOTE 6: Maximum number of repetitions shall depend upon the test configuration. NOTE 7: Cell ID shall depend upon the test configuration. |  |  |  |

### A.10.2.4	TDD in-band operation

Table A.10.2.4-1: NPDSCH Reference Channel for UE category NB1 for in-band operation

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| NPDSCH Reference channel | - | R.14 NB-TDD | R.15 NB-TDD |
| LTE Carrier bandwidth | MHz | 10 | 10 |
| Allocated resource blocks | PRB | 1 | 1 |
| Number of transmitter antennas | - | 1 | 2 |
| Allocated subframes per Radio Frame | subframes | Note 1 | Note 1 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |
| For Sub-Frame 4 | Bits | 40 Note 2 | 40 Note 2 |
| For Sub-Frame 0, 5, 9 | Bits | 0 | 0 |
| For Sub-Frame 1, 6 (DwPTS) | Bits | 16 or 40 Note 6 | 16 or 40 Note 6 |
| Number of Code Blocks per Sub-Frame |  |  |  |
| For Sub-Frame 4 |  | 1 Note 2 | 1 Note 2 |
| For Sub-Frame 0, 5, 9 |  | 0 | 0 |
| For Sub-Frame 1, 6 (DwPTS) |  | 1 | 1 |
| Maximum number of repetitions | - | Note 3 | Note 3 |
| NPDCCH start symbol | symbols | 3 | 3 |
| Cell ID | - | Note 4 | Note 4 |
| NOTE 1: Shall depend upon the NPDSCH scheduling. NOTE 2: Only apply for subframes scheduled with NPDSCH. NOTE 3: Maximum number of repetitions shall depend upon the test configuration. NOTE 4: Cell ID shall depend upon the test configuration. NOTE 5: SIB1-NB is transmitted in subframe 0 in odd radio frames. NOTE 6: Set 40 when NPDSCH is transmitted with repetition. Set 16 when NPDSCH is transmitted without repetition. |  |  |  |

### A.10.2.5	TDD standalone operation

Table A.10.2.5-1: NPDSCH Reference Channel for UE category NB1 for standalone operation

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| NPDSCH Reference channel | - | R.18 NB-TDD | R.19 NB-TDD |
| Channel bandwidth | KHz | 200 | 200 |
| Number of transmitter antennas | - | 1 | 2 |
| Allocated subframes per Radio Frame | subframes | Note 1 | Note 1 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |
| For Sub-Frame 4 | Bits | 72 Note 2 | 72 Note 2 |
| For Sub-Frame 0, 5, 9 | Bits | 0 | 0 |
| For Sub-Frame 1, 6 (DwPTS) | Bits | 40 or 72 Note 6 | 40 or 72 Note 6 |
| Number of Code Blocks per Sub-Frame |  |  |  |
| For Sub-Frame 4 |  | 1 Note 2 | 1 Note 2 |
| For Sub-Frame 0, 5, 9 |  | 0 | 0 |
| For Sub-Frame 1, 6 (DwPTS) |  | 1 | 1 |
| Maximum number of repetitions | - | Note 3 | Note 3 |
| Cell ID | - | Note 4 | Note 4 |
| NOTE 1: Shall depend upon the NPDSCH scheduling. NOTE 2: Only apply for subframes scheduled with NPDSCH. NOTE 3: Maximum number of repetitions shall depend upon the test configuration. NOTE 4: Cell ID shall depend upon the test configuration. NOTE 5: SIB1-NB is transmitted in subframe 0 in odd radio frames. NOTE 6: Set 72 when NPDSCH is transmitted with repetition. Set 40 when NPDSCH is transmitted without repetition. |  |  |  |

### A.10.2.6	TDD guard band operation

Table A.10.2.6-1: NPDSCH Reference Channel for UE category NB1 for guard band operation

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| NPDSCH Reference channel | - | R.22 NB-TDD | R.23 NB-TDD |
| LTE Carrier bandwidth | MHz | 10 | 10 |
| Allocated resource blocks for NB-IoT | PRB | 1 | 1 |
| Number of transmitter antennas | - | 1 | 2 |
| Allocated subframes per Radio Frame | subframes | Note 1 | Note 1 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |
| For Sub-Frame 4 | Bits | 72 Note 2 | 72 Note 2 |
| For Sub-Frame 0, 5, 9 | Bits | 0 | 0 |
| For Sub-Frame 1, 6 (DwPTS) | Bits | 40 or 72 Note 6 | 40 or 72 Note 6 |
| Number of Code Blocks per Sub-Frame |  |  |  |
| For Sub-Frame 4 |  | 1 Note 2 | 1 Note 2 |
| For Sub-Frame 0, 5, 9 |  | 0 | 0 |
| For Sub-Frame 1, 6 (DwPTS) |  | 1 | 1 |
| Maximum number of repetitions | - | Note 3 | Note 3 |
| Cell ID | - | Note 4 | Note 4 |
| NOTE 1: Shall depend upon the NPDSCH scheduling. NOTE 2: Only apply for subframes scheduled with NPDSCH. NOTE 3: Maximum number of repetitions shall depend upon the test configuration. NOTE 4: Cell ID shall depend upon the test configuration. NOTE 5: SIB1-NB is transmitted in subframe 0 in odd radio frames. NOTE 6: Set 72 when NPDSCH is transmitted with repetition. Set 40 when NPDSCH is transmitted without repetition. |  |  |  |

## A.10.3	Reference NPRACH Configurations

Table A.10.3-1 and A.10.3-2 defines the reference NB-IoT PRACH configurations for a NB-IoT RRM test case where the UE is required to transmit NPRACH during the testing procedure, but the testing purpose of the RRM test case does not include testing NPRACH performance.

Table A.10.3-1: NPRACH.R-1: HD-FDD Reference NPRACH Configuration

| Field | Value |  |  | Comment |
| --- | --- | --- | --- | --- |
| Prameters not per NPRACH coverage level |  |  |  |  |
| rsrp-ThresholdsPrach | {rsrp1, rsrp2} |  |  | The values of NPRACH RSRP thresholds for will be set according the requirement of individual test cases |
| nprach-CP-Length | us66dot7 |  |  | NPRACH format 0 |
| Parameters per NPRACH coverage Level |  |  |  |  |
| CE Level | Level 0 | Level 1 | Level 2 | Valid values as defined in 3GPP TS 36.331 [5] |
| nprach-Periodicity | ms40 | ms40 | ms40 | {ms40, ms80, ms160, ms240, ms320, ms640, ms1280, ms2560} |
| nprach-StartTime | ms8 | ms8 | ms8 | {ms8, ms16, ms32, ms64, ms128, ms256, ms512, ms1024} |
| nprach-SubcarrierOffset | n0 | n0 | n0 | {n0, n12, n24, n36, n2, n18, n34} |
| nprach-NumSubcarriers | n12 | n12 | n12 | {n12, n24, n36, n48} |
| nprach-SubcarrierMSG3-RangeStart | {one} | {one} | {one} | {zero, oneThird, twoThird, one} |
| maxNumPreambleAttemptCE | n3 | n5 | n7 | {n3, n4, n5, n6, n7, n8, n10} |
| numRepetitionsPerPreambleAttempt | n1 | n8 | n32 | {n1, n2, n4, n8, n16, n32, n64, n128} |
| npdcch-NumRepetitions-RA | r1 | r8 | r32 | {r1, r2, r4, r8, r16, r32, r64, r128, r256, r512, r1024, r2048} |
| npdcch-StartSF-CSS-RA | V8 | v2 | v2 | {v1dot5, v2, v4, v8, v16, v32, v48, v64} |
| npdcch-Offset-RA | zero | Zero | zero | {zero, oneEighth, oneFourth, threeEighth} |
| NOTE: See Clause 6.3.2 in 3GPP TS 36.331 [5] for further information on the parameters in this table. |  |  |  |  |

Table A.10.3-2: NPRACH.R-2: TDD Reference NPRACH Configuration

| Field | Value |  |  | Comment |
| --- | --- | --- | --- | --- |
| Prameters not per NPRACH coverage level |  |  |  |  |
| rsrp-ThresholdsPrach | {rsrp1, rsrp2} |  |  | The values of NPRACH RSRP thresholds for will be set according the requirement of individual test cases |
| nprach-PreambleFormat | fmt-0 |  |  | See 3GPP TS 36.211 [9] clause 10.1.6 |
| Parameters per NPRACH coverage Level |  |  |  |  |
| CE Level | Level 0 | Level 1 | Level 2 | Valid values as defined in 3GPP TS 36.331 [5] |
| nprach-Periodicity | ms80 | ms80 | ms80 | {ms80, ms160, ms320, ms640, ms1280, ms2560, ms5120, ms10240} |
| nprach-StartTime | ms10 | ms10 | ms10 | {ms10, ms20, ms40, ms80, ms160, ms320, ms640, ms1280, ms 2560, ms5120} |
| nprach-SubcarrierOffset | n0 | n0 | n0 | {n0, n12, n24, n36, n2, n18, n34} |
| nprach-NumSubcarriers | n12 | n12 | n12 | {n12, n24, n36, n48} |
| nprach-SubcarrierMSG3-RangeStart | {one} | {one} | {one} | {zero, oneThird, twoThird, one} |
| maxNumPreambleAttemptCE | n3 | n5 | n7 | {n3, n4, n5, n6, n7, n8, n10} |
| numRepetitionsPerPreambleAttempt | n1 | n8 | n32 | {n1, n2, n4, n8, n16, n32, n64, n128, n256, n512, n1024} |
| npdcch-NumRepetitions-RA | r1 | r8 | r32 | {r1, r2, r4, r8, r16, r32, r64, r128, r256, r512, r1024, r2048} |
| npdcch-StartSF-CSS-RA | v8 | v4 | v4 | {v4, v8, v16, v32, v48, v64, v96, v128} |
| npdcch-Offset-RA | zero | zero | zero | {zero, oneEighth, oneFourth, threeEighth} |
| nprach-NumCBRA-StartSubcarriers | n8 | n8 | n8 | {n8, n10, n11, n12, n20, n22, n23, n24, n32, n34, n35, n36, n40, n44, n48} |
| NOTE: See Clause 6.7.3.2 in 3GPP TS 36.331 [5] for further information on the parameters in this table. |  |  |  |  |

## A.10.4	Satellite specific parameters configuration

### A.10.4.1	General setup for SIB-31-NB

The general parameters for SIB31-NB setup is specified in Table A.10.4.1-1.

Table A.10.4.1-1: General setup for SIB31-NB

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| Interval between adjacent epoch time | s | LEO: 2.56 GEO, GSO: 10.24 |
| ntn-UlSyncValidityDuration | s | LEO: 5s GEO, GSO: 900s |
| k-Offset-r17 | ms | LEO: 14 GEO, GSO: 258 |
| kmac | ms | Not configured |
| ta-Common |  | 0 |
| ta-CommonDrift |  | 0 |
| ta-CommonDriftVariant |  | 0 |
| ephemerisInfo |  | According to TS 36.133 [5] Annex B.8 |
| ta-Report |  | Not configured |

### A.10.4.2	Satellite specific configuration for serving cell

The general parameters for SIB31-NB setup for serving satellite is specified in Table A.10.4.2-1.

Table A.10.4.2-1: SIB31-NB parameters setup for Serving satellite

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference configuration for serving satellite |  | SSC.1 | SSC.2 |
| Scenario |  | GSO or GEO (as per test definition) | NGSO |
| Interval between adjacent epoch time | s | 10.24 | 2.56 |
| ul-SyncValidityDuration-r17 | s | 900 | 5 |
| k-Offset-r17 | ms | 258 | 14 |
| k-Mac-r17 | ms | Not configured | Not configured |
| nta-Common-r17 |  | 0 | 0 |
| nta-CommonDrift-r17 |  | 0 | 0 |
| nta-CommonDriftVariation-r17 |  | 0 | 0 |
| ephemerisInfo |  | According to TS 36.133 [5] Annex B.8 |  |

### A.10.4.3	Satellite specific configuration for neighbour cell

The general parameters for SIB33-NB setup for neighbour cells in both serving and neighbour satellites are specified in Table A.10.4.3-1.

Table A.10.4.3-1: SIB33-NB parameters setup for neighbour cell

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference configuration for satellite assistance information for neighbour cell |  | NSC.1 | NSC.2 | NSC.3 | NSC.4 |
| Scenario |  | GSO or GEO (as per test definition) | NGSO | GSO or GEO (as per test definition) | NGSO |
| Interval between adjacent epoch time | s | 10.24 | 2.56 | 10.24 | 2.56 |
| neighValidityDuration-r18 | s | 900 | 5 | 900 | 5 |
| k-Mac-r18 | ms | Not configured | Not configured | Not configured | Not configured |
| nta-Common-r18 |  | 0 | 0 | 0 | 0 |
| nta-CommonDrift-r18 |  | 0 | 0 | 0 | 0 |
| nta-CommonDriftVariation-r18 |  | 0 | 0 | 0 | 0 |
| ephemerisInfo |  | Same as used for serving satellite |  | According to TS 36.133 [5] Annex B.8 NOTE1 |  |
| t-ServiceStartNeigh-r18 |  | NOTE 2 |  |  |  |
| Note 1: The ephemeris information for those neighbour cells shall be different than that used for the serving cell, i.e., these cells are associated to a different satellite. Note 2: Unless otherwise stated in the test case, t-serviceStartNeigh-r18 is not configured for the neighbour satellite. |  |  |  |  |  |

# A.11	V2V Sidelink Communication on Dedicated V2V Carrier

## A.11.1	Introduction

This clause also defines the principle and the reference configurations that are applicable to test cases verifying RRM core requirements for V2V sidelink communication on dedicated V2V carrier.

## A.11.2	Reference resource pool configurations for V2V Sidelink Communication

Table A.11.2-1: Pre-configuration for V2V Sidelink Communication

| Derivation Path: 3GPP TS 36.331 [3] clause 9.3.2, SL-V2X-Preconfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-V2X-PreconfigCommPool-r14 ::= SEQUENCE { |  |  |  |
| sl-OffsetIndicator-r14 | 0 | Indicates the offset of the first subframe of a resource pool within a SFN cycle. If absent, the resource pool starts from first subframe of SFN=0. |  |
| sl-Subframe-r14 included in SL-PreconfigV2X-TxPoolList | 10000000000000000000 10000000000000000000 10000000000000000000 10000000000000000000 10000000000000000000 | Indicates the bitmap of the TX resource pool, which is defined by repeating the bitmap within a SFN cycle (see 3GPP TS 36.213 [23]) |  |
| sl-Subframe-r14 included in SL-PreconfigV2X-RxPoolList | 11111111111111111111 11111111111111111111 11111111111111111111 11111111111111111111 11111111111111111111 | Indicates the bitmap of the RX resource pool, which is defined by repeating the bitmap within a SFN cycle (see 3GPP TS 36.213 [23]) |  |
| adjacencyPSCCH-PSSCH-r14 | True | Adjacent: TRUE Non-adjacent: FALSE |  |
| sizeSubchannel-r14 | 5 | Minimum bandwidth of subchannel for adjacent transmission |  |
| startRB-Subchannel-r14 | 0 | Indicates the lowest RB index of the subchannel with the lowest index. |  |
| startRB-PSCCH-Pool-r14 | 0 | Indicates the lowest RB index of the PSCCH pool |  |
| } |  |  |  |

## A.11.3	Reference measurement channels for V2V Sidelink Communication

Table A.11.3-1: PSCCH Reference Measurement Channels

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Reference channel |  |  | CC.1 |
| Channel bandwidth |  | MHz | 10 |
| Allocated PSCCH resource blocks |  |  | 2 |
| DFT-OFDM symbols per subframe (see Note 1) |  |  | 9 |
| Modulation |  |  | QPSK |
| Information Bit Payload (without CRC) |  | Bits | 32 |
| Information Bit | SCI Format |  | 1 |
|  | Priority |  | As set by higher layers |
|  | Resource reservation |  | 0 |
|  | Modulation and coding scheme |  | Set as the PSSCH MCS specified in the test |
|  | Retransmission index |  | Note 4 |
|  | Time gap between initial transmission and retransmission |  | 0 (Note 2) |
|  | Frequency resource location of the initial transmission and retransmission |  | Initial transmission: Set as per PSSCH RB allocation specific in the test Retransmission: Note 4 |
|  | Reserved bits |  | Set all these bits to 0 |
| Transport block CRC |  | Bits | 16 |
| Binary Channel Bits (see Note 3) |  | Bits | 432 |
| NOTE 1: PSCCH transmissions are rate-matched for 10 DFT-OFDM symbols per subframe, and the last symbol shall be punctured as per 3GPP TS 36.211. NOTE 2: SFgap is the value indicated by "Time gap between initial transmission and retransmission" field in the configured sidelink grant, and SFgap =0 means no retransmission of the associated TB as per 3GPP TS 36.213. NOTE 3: Binary channel bits calculated under assumption of 9 DFT-OFDM symbols per subframe. NOTE 4: UE is allowed to autonomously select the un-used or redundant bits/code-points in SCI format 1. |  |  |  |

Table A.11.3-2: PSSCH Reference Measurement Channels

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | CD.1 |
| Sidelink transmission mode |  | 4 |
| Channel bandwidth | MHz | 10 |
| Allocated PSSCH resource blocks |  | 3 |
| DFT-OFDM symbols per subframe (see Note 1) |  | 9 |
| Modulation |  | QPSK |
| Target Code Rate |  | 1/3 |
| Information Bit Payload (Transport block size) | Bits | 208 |
| Transport block CRC | Bits | 24 |
| Number of PSSCH HARQ retransmissions |  | 0 |
| Binary Channel Bits (see Note 2) | Bits | 648 |
| NOTE 1: PSSCH transmissions are rate-matched for 10 DFT-OFDM symbols per subframe, and the last symbol shall be punctured as per 3GPP TS 36.211. NOTE 2: Binary channel bits calculated under assumption of 9 DFT-OFDM symbols per subframe. |  |  |

# A.12	V2X sidelink communication

## A.12.1	Introduction

This clause also defines the principle and the reference configurations that are applicable to test cases verifying RRM core requirements for V2X sidelink communication.

## A.12.2	Reference resource pool configurations for V2X Sidelink Communication

Table A.12.2-1: Pre-configuration for V2X Sidelink Communication (Configuration #1)

| Derivation Path: 3GPP TS 36.331 [3] clause 9.3.2, SL-V2X-Preconfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-V2X-PreconfigCommPool-r14 ::= SEQUENCE { |  |  |  |
| sl-OffsetIndicator-r14 | 0 | Indicates the offset of the first subframe of a resource pool within a SFN cycle. If absent, the resource pool starts from first subframe of SFN=0. |  |
| sl-Subframe-r14 included in SL-PreconfigV2X-TxPoolList | 11111111111111111111 11111111111111111111 11111111111111111111 11111111111111111111 11111111111111111111 | Indicates the bitmap of the TX resource pool, which is defined by repeating the bitmap within a SFN cycle (see 3GPP TS 36.213 [23]) |  |
| sl-Subframe-r14 included in SL-PreconfigV2X-RxPoolList | 11111111111111111111 11111111111111111111 11111111111111111111 11111111111111111111 11111111111111111111 | Indicates the bitmap of the RX resource pool, which is defined by repeating the bitmap within a SFN cycle (see 3GPP TS 36.213 [23]) |  |
| adjacencyPSCCH-PSSCH-r14 | True | Adjacent: TRUE Non-adjacent: FALSE |  |
| sizeSubchannel-r14 | 5 | Minimum bandwidth of subchannel for adjacent transmission |  |
| startRB-Subchannel-r14 | 0 | Indicates the lowest RB index of the subchannel with the lowest index. |  |
| startRB-PSCCH-Pool-r14 | 0 | Indicates the lowest RB index of the PSCCH pool |  |
| } |  |  |  |

Table A.12.2-2: V2X sidelink Communication configuration for E-UTRAN (Configuration #2)

| Derivation Path: 3GPP TS 36.331 clause 6.3.8 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  |  | Value (10MHz) | Comment |
| SL-V2X-InterFreqUE-Config-r14 ::= SEQUENCE { |  |  |  |  |
|  | physCellIdList-r14 |  |  | Not present |
|  | typeTxSync-r14 |  | Set according to the specific test configuration | ENUMERATED {gnss, enb, ue} |
|  | v2x-SyncConfig-r14 |  |  | Not present |
|  | v2x-CommRxPool-r14 | SL-CommResourcePoolV2X -r14 SEQUENCE { |  | RxPool A monitoring UE can receive on the resources of this pool when a transmitting UE uses the Tx Pool Normal or Tx Pool Exceptional |
|  |  | sl-OffsetIndicator-r14 | 0 | small-r12 Indicates the offset of the first subframe of a resource pool within a SFN cycle. If absent, the resource pool starts from first subframe of SFN=0. |
|  |  | sl-Subframe-r14 | 11111111 11111111 1111 | bs20-r14 for FDD Indicates the bitmap of the TX resource pool, which is defined by repeating the bitmap within a SFN cycle (see 3GPP TS 36.213 [23]) |
|  |  | adjacencyPSCCH-PSSCH-r14 | TRUE | BOOLEAN Adjacent: TRUE Non-adjacent: FALSE |
|  |  | sizeSubchannel-r14 | 50 | ENUMERATED {n50} Minimum bandwidth of subchannel for adjacent transmission |
|  |  | numSubchannel-r14 | 1 | ENUMERATED {n1} Number of subchannel for adjacent transmission |
|  |  | startRB-Subchannel-r14 | 0 | Indicates the lowest RB index of the subchannel with the lowest index. |
|  |  | startRB-PSCCH-Pool-r14 | 0 | Indicates the lowest RB index of the PSCCH pool |
|  |  | } |  |  |
|  | v2x-CommTxPoolNormal-r14 | SL-CommResourcePoolV2X -r14 SEQUENCE { |  | Tx Pool Normal |
|  |  | sl-OffsetIndicator-r14 | 0 | small-r12 |
|  |  | sl-Subframe-r14 | 11111111 11111111 0000 | bs20-r14 |
|  |  | adjacencyPSCCH-PSSCH-r14 | TRUE | BOOLEAN |
|  |  | sizeSubchannel-r14 | 50 | ENUMERATED {n50} |
|  |  | numSubchannel-r14 | 1 | ENUMERATED {n1} |
|  |  | startRB-Subchannel-r14 | 0 |  |
|  |  | startRB-PSCCH-Pool-r14 | 0 |  |
|  |  | dataTxParameters-r14 SEQUENCE { |  |  |
|  |  | alpha-r12 | al0 | Sidelink power control: al0 corresponds to 0 |
|  |  | p0-r12 | 31 | INTEGER (-126..31), unit dBm |
|  |  | } |  |  |
|  |  | } |  |  |
|  | v2x-CommTxPoolExceptional-r14 | SL-CommResourcePoolV2X -r14 SEQUENCE { |  | Tx Pool Exceptional |
|  |  | sl-OffsetIndicator-r14 | 0 | small-r12 |
|  |  | sl-Subframe-r14 | 00000000 00000000 1111 | bs20-r14 |
|  |  | adjacencyPSCCH-PSSCH-r14 | TRUE | BOOLEAN |
|  |  | sizeSubchannel-r14 | 50 | ENUMERATED {n50} |
|  |  | numSubchannel-r14 | 1 | ENUMERATED {n1} |
|  |  | startRB-Subchannel-r14 | 0 |  |
|  |  | startRB-PSCCH-Pool-r14 | 0 |  |
|  |  | dataTxParameters-r14 SEQUENCE { |  |  |
|  |  | alpha-r12 | al0 | Sidelink power control: al0 corresponds to 0 |
|  |  | p0-r12 | 31 | INTEGER (-126..31), unit dBm |
|  |  | } |  |  |
|  |  | } |  |  |
|  | p2x-CommTxPoolNormal-r14 |  |  | Not present |
|  | v2x-ResourceSelectionConfig-r14 | SL-CommTxPoolSensingConfig-r14::= SEQUENCE { |  |  |
|  |  | pssch-TxConfigList-r14 ::= SEQUENCE { | SL-PSSCH-TxConfig-r14 |  |
|  |  | typeTxSync-r14 | Set according to the specific test configuration | ENUMERATED {gnss, enb, ue} |
|  |  | thresUE-Speed-r14 | kmph200 |  |
|  |  | parametersAboveThres-r14 SEQUENCE { |  |  |
|  |  | minMCS-PSSCH-r14 | 0 |  |
|  |  | maxMCS-PSSCH-r14 | 15 |  |
|  |  | minSubChannel-NumberPSSCH-r14 | 1 |  |
|  |  | maxSubchannel-NumberPSSCH-r14 | 1 |  |
|  |  | allowedRetxNumberPSSCH-r14 | Both |  |
|  |  | maxTxPower-r14 | Not present |  |
|  |  | } |  |  |
|  |  | parametersBelowThres-r14 SEQUENCE { |  |  |
|  |  | minMCS-PSSCH-r14 | 4 |  |
|  |  | maxMCS-PSSCH-r14 | 25 |  |
|  |  | minSubChannel-NumberPSSCH-r14 | 1 |  |
|  |  | maxSubchannel-NumberPSSCH-r14 | 1 |  |
|  |  | allowedRetxNumberPSSCH-r14 | n1 |  |
|  |  | maxTxPower-r14 | Not present |  |
|  |  | } |  |  |
|  |  | } |  |  |
|  |  | thresPSSCH-RSRP-List-r14 SEQUENCE (SIZE (64)) OF SL-ThresPSSCH-RSRP-r14 { |  |  |
|  |  | SL-ThresPSSCH-RSRP-r14[n] | 1 | For n=1,2,…,64, where n denotes the index for the threshold used for sensing based UE autonomous resource selection |
|  |  | } |  |  |
|  |  | restrictResourceReservationPeriod-r14 | {v1} | BIT STRING (SIZE (10)) |
|  |  | probResourceKeep-r14 | v0 |  |
|  |  | } |  |  |
|  | zoneConfig-r14 |  |  | Not present |
| } |  |  |  |  |

## A.12.3	Reference measurement channels for V2X Sidelink Communication

Table A.12.3-1: PSCCH Reference Measurement Channels

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Reference channel |  |  | CC.1A HD |
| Channel bandwidth |  | MHz | 10 |
| Allocated PSCCH resource blocks |  |  | 2 |
| DFT-OFDM symbols per subframe (see Note 1) |  |  | 9 |
| Modulation |  |  | QPSK |
| Information Bit Payload (without CRC) |  | Bits | 32 |
| Information Bit | SCI Format |  | 1 |
|  | Priority |  | As set by higher layers |
|  | Resource reservation |  | 0 |
|  | Modulation and coding scheme |  | Set as the PSSCH MCS specified in the test |
|  | Retransmission index |  | Note 4 |
|  | Time gap between initial transmission and retransmission |  | 0 (Note 3) |
|  | Frequency resource location of the initial transmission and retransmission |  | Initial transmission: Set as per PSSCH RB allocation specific in the test Retransmission: Note 4 |
|  | Reserved bits |  | Set all these bits to 0 |
| Transport block CRC |  | Bits | 16 |
| Binary Channel Bits (see Note 2) |  | Bits | 432 |
| NOTE 1: PSCCH transmissions are rate-matched for 10 DFT-OFDM symbols per subframe, and the last symbol shall be punctured as per 3GPP TS 36.211. NOTE 2: Binary channel bits calculated under assumption of 9 DFT-OFDM symbols per subframe. NOTE 3: SFgap is the value indicated by "Time gap between initial transmission and retransmission" field in the configured sidelink grant, and SFgap =0 means no retransmission of the associated TB as per 3GPP TS 36.213. NOTE 4: UE is allowed to autonomously select the un-used or redundant bits/code-points in SCI format 1 |  |  |  |

Table A.12.3-2: PSSCH Reference Measurement Channels

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | CD.1A HD |
| Sidelink transmission mode |  | 4 |
| Channel bandwidth | MHz | 10 |
| Allocated PSSCH resource blocks |  | 48 |
| DFT-OFDM symbols per subframe (see Note 1) |  | 9 |
| Modulation |  | QPSK |
| Target Code Rate |  | 1/3 |
| Information Bit Payload (Transport block size) | Bits | 3496 |
| Transport block CRC | Bits | 24 |
| Number of PSSCH HARQ retransmissions |  | 0 |
| Binary Channel Bits (see Note 2) | Bits | 10368 |
| NOTE 1: PSSCH transmissions are rate-matched for 10 DFT-OFDM symbols per subframe, and the last symbol shall be punctured as per 3GPP TS 36.211. NOTE 2: Binary channel bits calculated under assumption of 9 DFT-OFDM symbols per subframe. |  |  |

Table A.12.3-3: PSSCH Reference Measurement Channels

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | CD.1B HD |
| Sidelink transmission mode |  | 4 |
| Channel bandwidth | MHz | 10 |
| Allocated PSSCH resource blocks |  | 3 |
| DFT-OFDM symbols per subframe (see Note 1) |  | 9 |
| Modulation |  | QPSK |
| Target Code Rate |  | 1/3 |
| Information Bit Payload (Transport block size) | Bits | 208 |
| Transport block CRC | Bits | 24 |
| Number of PSSCH HARQ retransmissions |  | 0 |
| Binary Channel Bits (see Note 2) | Bits | 648 |
| NOTE 1: PSSCH transmissions are rate-matched for 10 DFT-OFDM symbols per subframe, and the last symbol shall be punctured as per 3GPP TS 36.211. NOTE 2: Binary channel bits calculated under assumption of 9 DFT-OFDM symbols per subframe. |  |  |

Annex B (normative):
Propagation Conditions

# B.0	No interference

See 3GPP TS 36.521-1 [10] Annex B.0.

# B.1	Static propagation condition

See 3GPP TS 36.521-1 [10] Annex B.1 and B.1.1

# B.2	Multi-path fading Propagation Conditions

See 3GPP TS 36.521-1 [10] Annex B.2,B.2.1 and B.2.2

Annex C (normative):
Downlink Physical Channels

# C.0	Downlink signal

See 3GPP TS 36.521-1 [10] Annex C.0.

# C.1	General

See 3GPP TS 36.521-1 [10] Annex C.1.

For CA tests with 1 UL, no PHICH resources are assigned to the DUT in the SCell. In such case PHICH of SCell is entirely part of OCNG pattern used.

# C.2	Set-up

See 3GPP TS 36.521-1 [10] Annex C.2.

# C.3	Test specific scenarios

## C.3.1	ABS Transmission Configurations

### C.3.1.1	Non-MBSFN ABS Transmission Configurations

#### C.3.1.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH

Table C.3.1.1.1-1: Transmission configuration with non-MBSFN ABS, 1x2 with PBCH

| Physical Channels and Signals | Parameters | EPRE, [dB] |  |
| --- | --- | --- | --- |
|  |  | Non-ABS | ABS |
| PBCH | PBCH_RA | 0 | 0 |
|  | PBCH_RB | 0 | 0 |
| PSS | PSS_RA | 0 | 0 |
| SSS | SSS_RA | 0 | 0 |
| PCFICH | PCFICH_RB | 0 | 0 (Note 1) |
| PHICH | PHICH_RA | 0 | -Inf |
|  | PHICH_RB | 0 | -Inf |
| PDCCH | PDCCH_RA | 0 | 0 (Note 1) |
|  | PDCCH_RB | 0 | 0 (Note 1) |
| PDSCH | PDSCH_RA | 0 | 0 (Note 1) |
|  | PDSCH_RB | 0 | 0 (Note 1) |
| OCNG | OCNG_RA | 0 | -Inf |
|  | OCNG_RB | 0 | -Inf |
| NOTE 1: Only used for SIB1, otherwise EPRE is -Inf. NOTE 2: 1x2 antenna configuration is assumed. |  |  |  |

#### C.3.1.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH

Table C.3.1.1.2-1: Transmission configuration #1 with non-MBSFN ABS, 2x2 without PBCH

| Physical Channels and Signals | Parameters | EPRE [dB] |  |
| --- | --- | --- | --- |
|  |  | Non-ABS | ABS |
| PBCH | PBCH_RA | -3 | -Inf |
|  | PBCH_RB | -3 | -Inf |
| PSS | PSS_RA | -3 | -3 |
| SSS | SSS_RA | -3 | -3 |
| PCFICH | PCFICH_RB | 1 | -Inf |
| PHICH | PHICH_RA | -3 | -Inf |
|  | PHICH_RB | -3 | -Inf |
| PDCCH | PDCCH_RA | 1 | -Inf |
|  | PDCCH_RB | 1 | -Inf |
| PDSCH | PDSCH_RA | -3 | -Inf |
|  | PDSCH_RB | -3 | -Inf |
| OCNG | OCNG_RA | -3 | -Inf |
|  | OCNG_RB | -3 | -Inf |
| NOTE: 2x2 antenna configuration is assumed |  |  |  |

Table C.3.1.1.2-2: Transmission configuration #2 with non-MBSFN ABS, 2x2 without PBCH

| Physical Channels and Signals | Parameters | EPRE [dB] |  |
| --- | --- | --- | --- |
|  |  | Non-ABS | ABS |
| PBCH | PBCH_RA | -3 | -Inf |
|  | PBCH_RB | -3 | -Inf |
| PSS | PSS_RA | -3 | -3 |
| SSS | SSS_RA | -3 | -3 |
| PCFICH | PCFICH_RB | 1 | -Inf |
| PHICH | PHICH_RA | -3 | -Inf |
|  | PHICH_RB | -3 | -Inf |
| PDCCH | PDCCH_RA | -3 | -Inf |
|  | PDCCH_RB | -3 | -Inf |
| PDSCH | PDSCH_RA | -3 | -Inf |
|  | PDSCH_RB | -3 | -Inf |
| OCNG | OCNG_RA | -3 | -Inf |
|  | OCNG_RB | -3 | -Inf |
| NOTE: 2x2 antenna configuration is assumed |  |  |  |

### C.3.1.2	MBSFN ABS Transmission Configurations

#### C.3.1.2.1	MBSFN ABS Transmission, 1x2 antenna

Table C.3.1.2.1-1: Transmission configuration with MBSFN ABS, 1x2

| Physical Channels and Signals | Parameters | EPRE, [dB] |  |
| --- | --- | --- | --- |
|  |  | Non-ABS | ABS |
| PBCH | PBCH_RA | 0 | N/A |
|  | PBCH_RB | 0 | N/A |
| PSS | PSS_RA | 0 | N/A |
| SSS | SSS_RA | 0 | N/A |
| PCFICH | PCFICH_RB | 0 | -Inf |
| PHICH | PHICH_RA | 0 | -Inf |
|  | PHICH_RB | 0 | -Inf |
| PDCCH | PDCCH_RA | 0 | -Inf |
|  | PDCCH_RB | 0 | -Inf |
| PDSCH | PDSCH_RA | 0 | -Inf |
|  | PDSCH_RB | 0 | -Inf |
| PMCH | PMCH_RA | 0 | -Inf |
|  | PMCH_RB | 0 | -Inf |
| OCNG | OCNG_RA | 0 | -Inf |
|  | OCNG_RB | 0 | -Inf |
| NOTE: 1x2 antenna configuration is assumed |  |  |  |

#### C.3.1.2.2	MBSFN ABS Transmission, 2x2 antenna

Table C.3.1.2.2-1: Transmission configuration #1 with MBSFN ABS, 2x2

| Physical Channels and Signals | Parameters | EPRE, [dB] |  |
| --- | --- | --- | --- |
|  |  | Non-ABS | ABS |
| PBCH | PBCH_RA | -3 | N/A |
|  | PBCH_RB | -3 | N/A |
| PSS | PSS_RA | -3 | N/A |
| SSS | SSS_RA | -3 | N/A |
| PCFICH | PCFICH_RB | 1 | -Inf |
| PHICH | PHICH_RA | -3 | -Inf |
|  | PHICH_RB | -3 | -Inf |
| PDCCH | PDCCH_RA | 1 | -Inf |
|  | PDCCH_RB | 1 | -Inf |
| PDSCH | PDSCH_RA | -3 | -Inf |
|  | PDSCH_RB | -3 | -Inf |
| PMCH | PMCH_RA | -3 | -Inf |
|  | PMCH_RB | -3 | -Inf |
| OCNG | OCNG_RA | -3 | -Inf |
|  | OCNG_RB | -3 | -Inf |
| NOTE: 2x2 antenna configuration is assumed |  |  |  |

Table C.3.1.2.2-2: Transmission configuration # 2 with MBSFN ABS, 2x2

| Physical Channels and Signals | Parameters | EPRE, [dB] |  |
| --- | --- | --- | --- |
|  |  | Non-ABS | ABS |
| PBCH | PBCH_RA | -3 | N/A |
|  | PBCH_RB | -3 | N/A |
| PSS | PSS_RA | -3 | N/A |
| SSS | SSS_RA | -3 | N/A |
| PCFICH | PCFICH_RB | 1 | -Inf |
| PHICH | PHICH_RA | -3 | -Inf |
|  | PHICH_RB | -3 | -Inf |
| PDCCH | PDCCH_RA | -3 | -Inf |
|  | PDCCH_RB | -3 | -Inf |
| PDSCH | PDSCH_RA | -3 | -Inf |
|  | PDSCH_RB | -3 | -Inf |
| PMCH | PMCH_RA | -3 | -Inf |
|  | PMCH_RB | -3 | -Inf |
| OCNG | OCNG_RA | -3 | -Inf |
|  | OCNG_RB | -3 | -Inf |
| NOTE: 2x2 antenna configuration is assumed |  |  |  |

## C.3.2	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases

### C.3.2.1	Impact of Reference Sensitivity Degradation due to Insertion Loss

For a UE supporting inter-band carrier aggregation configuration with uplink in one E-UTRA band, if there is a relaxation of receiver sensitivity ΔRIB,c>0 dB as defined in 3GPP TS 36.101 [2], 7.3.1-1A, there is no adjustment of test parameters in the tests specified in 3GPP TS 36.133 when ΔRIB,c  1 dB.

## C.3.3	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations

### C.3.3.1	Introduction

In clauses 7, 8 and 9 carrier aggregation test cases may be defined with different channel bandwidth combinations to verify the same RRM requirement.

If multiple carrier aggregation test cases with different channel bandwidth combinations are defined to verify the same RRM requirement that is channel bandwidth independent, then the UE needs to be tested only with one bandwidth combination out of the bandwidth combination sets supported by that UE.

## C.3.4	Proximity-based Services

### C.3.4.1	Introduction

This clause also defines the principle and the reference configurations that are applicable to test cases verifying RRM core requirements for ProSe Direct Discovery and ProSe Direct Communication.

### C.3.4.2	Reference DRX configurations for ProSe tests

Table C.3.4.2-1: Reference DRX Configurations

| Parameter | Value |
| --- | --- |
| Reference configuration | DRX_P1 |
| onDurationTimer | psf1 |
| drx-InactivityTimer | psf1 |
| drx-RetransmissionTimer | psf1 |
| longDRX-CycleStartOffset | sf320, 0 |
| shortDRX | Disabled |
| NOTE: For further information see clause 6.3.2 in 3GPP TS 36.331. |  |

### C.3.4.3	Test Cases with Different Channel Bandwidths

#### C.3.4.3.1	Introduction

This clause defines a principle which is applicable to test cases verifying ProSe RRM requirements with different channel bandwidths.

#### C.3.4.3.2	Principle of testing

Some ProSe test cases are defined for different channel bandwidths to verify the same RRM requirement.

If test cases with different channel bandwidth are defined to verify the same RRM requirement then the UE is required to pass the test cases only with one of the channel bandwidths.

## C.3.5	Listen before talk model

### C.3.5.1	Introduction

In some RRM test cases for FS3, a Listen Before Talk (LBT) model is specified. The intention of the LBT model is to emulate using test equipment the behaviour of an FS3 eNB which performs channel measurement to check that the channel is clear prior to performing downlink transmission.

### C.3.5.2	Definition

Prior to each DMTC window, the test equipment shall determine whether to transmit a Discovery Reference Signal (DRS) during the DMTC window with probability P=0.75. In many cases the test requirement depends on the number of configured discovery signal occasions which are not available during the test, so the test equipment shall track how many DRS are not transmitted during the test period. If the test equipment determines that it shall transmit a DRS, then the timing of the DRS transmission within the DMTC window is randomly selected from the set of possible DRS transmission signal timings, such that there is an equal probability of any valid DRS timing.

For non DRS downlink transmission bursts,  if transmission occurred in the previous subframe, transmission is muted for a duration of one subframe. Additionally, if the start time of the candidate transmission burst is within [8] subframes of the start of the DMTC window, transmission is not performed. Otherwise

The length of the transmission burst in subframes is defined as N. The burst transmission format is determined according to the steps below:

1)	Select N randomly from a given set of the number of subframes S1={1,3,5,8} with equal probability as the total length of burst transmission format.

2)	A uniform random variable from 0 to 1 is generated. If the random variable is less than P=0.75, a burst of N fully occupied subframes is transmitted. Otherwise, the burst transmission is muted and the muting duration is the same as the number N of subframes for determined burst format.

Annex D (normative):
OFDMA Channel Noise Generator (OCNG)

# D.1	OCNG Patterns for FDD

## D.0

The following OCNG patterns are used for modelling allocations to virtual UEs (which are not under test) and/or allocations used for MBSFN. The OCNG pattern for each sub frame specifies the allocations that shall be filled with OCNG, and furthermore, the relative power level of each such allocation.

In each test case the OCNG is expressed by parameters OCNG_RA and OCNG_RB which together with a relative power level () specifies the PDSCH EPRE-to-RS EPRE ratios in OFDM symbols with and without reference symbols, respectively. The relative power, which is used for modelling boosting per virtual UE allocation, is expressed by:

where  denotes the relative power level of the i:th virtual UE. The parameter settings of OCNG_RA, OCNG_RB, and the set of relative power levels are chosen such that when also taking allocations to the UE under test into account, as given by a PDSCH reference channel, a constant transmitted power spectral density that is constant on an OFDM symbol basis is targeted.

Moreover the OCNG pattern is accompanied by a PCFICH/PDCCH/PHICH reference channel which specifies the control region. The number of PDCCH OFDM symbols in the non-MBSFN subframes is the same as specified in the RMC used in the test. The number of PDCCH OFDM symbols in the MBSFN subframes is the maximal allowed according to 3GPP TS 36.213 [8]. For any aggregation and PHICH allocation, the PDCCH and any unused PHICH groups are padded with resource element groups with a power level given respectively by PDCCH_RA/RB and PHICH_RA/RB as specified in the test case such that a total power spectral density in the control region that is constant on an OFDM symbol basis is targeted.

For subframes configured as PRS subframes the PDSCH allocation defined in the OCNG pattern does not apply.

For subframes configured as ABS subframes the PDSCH and PMCH allocation defined in the OCNG pattern does not apply.

The system information is scheduled in the allocations reserved for the OCNG patterns, in the subframes not configured for MBSFN. For this purpose the number of the RB-s allocated with PDSCH defined in the OCNG pattern can be reduced as necessary.

## D.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz

Table D.1.1-1: OP.1 FDD: OCNG FDD Pattern 1

| Allocation | Relative power level [dB] |  |  |  | PDSCH Data | PMCH Data |
| --- | --- | --- | --- | --- | --- | --- |
|  | Subframe |  |  |  |  |  |
|  | 0 | 5 | 4,9 | 1-3, 6-8 |  |  |
| 0 - 12 | 0 | 0 | 0 | N/A | Note 1 | N/A |
| 37 - 49 | 0 | 0 | 0 | N/A |  |  |
| 0-49 | N/A | N/A | N/A | Note 4 | N/A | Note 2 |
| NOTE 1: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameteris used to scale the power of PDSCH. NOTE 2: Each physical resource block (PRB) is assigned to MBSFN transmission. The data in each PRB shall be uncorrelated with data in other PRBs over the period of any measurement. The MBSFN data shall be QPSK modulated. PMCH symbols shall not contain cell-specific Reference Signals. PMCH subframes shall contain cell-specific Reference Signals only in the first symbol of the first time slot. The parameteris used to scale the power of PMCH. NOTE 3: If two or more transmit antennas with CRS are used in the test, the PDSCH part of OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the PDSCH part of OCNG is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. NOTE 4: 0 dB for 1 transmit antenna with CRS, +3 dB for 2 transmit antennas with CRS. NOTE 5: If SPDCCH is to be transmitted on a physical resource block (PRB), this takes priority over OCNG transmission. N/A: Not Applicable. |  |  |  |  |  |  |

## D.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz

Table D.1.2-1: OP.2 FDD: OCNG FDD Pattern 2

| Allocation | Relative power level [dB] |  |  |  | PDSCH Data | PMCH Data |
| --- | --- | --- | --- | --- | --- | --- |
|  | Subframe |  |  |  |  |  |
|  | 0 | 5 | 4, 9 | 1 - 3, 6 - 8 |  |  |
| 0 - 49 | 0 | 0 | 0 | N/A | Note 1 | N/A |
| 0 - 49 | N/A | N/A | N/A | Note 4 | N/A | Note 2 |
| NOTE 1: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameteris used to scale the power of PDSCH. NOTE 2: Each physical resource block (PRB) is assigned to MBSFN transmission. The data in each PRB shall be uncorrelated with data in other PRBs over the period of any measurement. The MBSFN data shall be QPSK modulated. PMCH symbols shall not contain cell-specific Reference Signals. PMCH subframes shall contain cell-specific Reference Signals only in the first symbol of the first time slot. The parameteris used to scale the power of PMCH. NOTE 3: If two or more transmit antennas with CRS are used in the test, the PDSCH part of OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the PDSCH part of OCNG is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. NOTE 4: 0 dB for 1 transmit antenna with CRS, +3 dB for 2 transmit antennas with CRS. N/A: Not Applicable. |  |  |  |  |  |  |

## D.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz

Table A.3.2.1.3-1: OP.3 FDD: OCNG FDD Pattern 3

| Allocation | Relative power level [dB] |  |  |  | PDSCH Data | PMCH Data |
| --- | --- | --- | --- | --- | --- | --- |
|  | Subframe |  |  |  |  |  |
|  | 0 | 5 | 4,9 | 1-3, 6-8 |  |  |
| 0 - 1 | 0 | 0 | 0 | N/A | Note 1 | N/A |
| 4 - 5 | 0 | 0 | 0 | N/A |  |  |
| 0 - 5 | N/A | N/A | N/A | Note 4 | N/A | Note 2 |
| NOTE 1: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameteris used to scale the power of PDSCH. NOTE 2: Each physical resource block (PRB) is assigned to MBSFN transmission. The data in each PRB shall be uncorrelated with data in other PRBs over the period of any measurement. The MBSFN data shall be QPSK modulated. PMCH symbols shall not contain cell-specific Reference Signals. PMCH subframes shall contain cell-specific Reference Signals only in the first symbol of the first time slot. The parameteris used to scale the power of PMCH. NOTE 3: If two or more transmit antennas with CRS are used in the test, the PDSCH part of OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the PDSCH part of OCNG is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. NOTE 4: 0 dB for 1 transmit antenna with CRS, +3 dB for 2 transmit antennas with CRS. N/A: Not Applicable. |  |  |  |  |  |  |

## D.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz

Table A.3.2.1.4-1: OP.4 FDD: OCNG FDD Pattern 4

| Allocation | Relative power level [dB] |  |  |  | PDSCH Data | PMCH Data |
| --- | --- | --- | --- | --- | --- | --- |
|  | Subframe |  |  |  |  |  |
|  | 0 | 5 | 4, 9 | 1 - 3, 6 - 8 |  |  |
| 0 - 5 | 0 | 0 | 0 | N/A | Note 1 | N/A |
| 0 - 5 | N/A | N/A | N/A | Note 4 | N/A | Note 2 |
| NOTE 1: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameteris used to scale the power of PDSCH. NOTE 2: Each physical resource block (PRB) is assigned to MBSFN transmission. The data in each PRB shall be uncorrelated with data in other PRBs over the period of any measurement. The MBSFN data shall be QPSK modulated. PMCH subframes shall contain cell-specific Reference Signals only in the first symbol of the first time slot. The parameteris used to scale the power of PMCH. NOTE 3: If two or more transmit antennas with CRS are used in the test, the PDSCH part of OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the PDSCH part of OCNG is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. NOTE 4: 0 dB for 1 transmit antenna with CRS, +3 dB for 2 transmit antennas with CRS. N/A: Not Applicable. |  |  |  |  |  |  |

## D.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)

Table D.1.5-1: OP.5 FDD: OCNG FDD Pattern 5

| Allocation | Relative power level [dB] |  |  |  | PDSCH Data |
| --- | --- | --- | --- | --- | --- |
|  | Subframe (Note 1) |  |  |  |  |
|  | 0 | 5 | 4, 9 | 1 - 3, 6 - 8 |  |
| 0 - 12 | 0 | 0 | 0 | N/A | Note 2 |
| 37 - 49 | 0 | 0 | 0 | N/A |  |
| 0 - 49 | N/A | N/A | N/A | 0 |  |
| NOTE 1: The allocation of any PDSCH with or without SIB1 applies only to the subframes not configured as PRS subframes. NOTE 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameter is used to scale the power of PDSCH. NOTE 3: If two or more transmit antennas with CRS are used in the test, the PDSCH part of OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the PDSCH part of OCNG is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. N/A: Not Applicable. |  |  |  |  |  |

## D.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)

Table D.1.6-1: OP.6 FDD: OCNG FDD Pattern 6

| Allocation | Relative power level [dB] |  |  |  | PDSCH Data |
| --- | --- | --- | --- | --- | --- |
|  | Subframe (Note 1) |  |  |  |  |
|  | 0 | 5 | 4, 9 | 1 - 3, 6 - 8 |  |
| 0 - 49 | 0 | 0 | 0 | 0 | Note 2 |
| NOTE 1: The allocation of any PDSCH with or without SIB1 applies only to the subframes not configured as PRS subframes. NOTE 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameter is used to scale the power of PDSCH. NOTE 3: If two or more transmit antennas with CRS are used in the test, the PDSCH part of OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the PDSCH part of OCNG is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. N/A: Not Applicable. |  |  |  |  |  |

## D.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)

Table D.1.7-1: OP.7 FDD: OCNG FDD Pattern 7

| Allocation | Relative power level [dB] |  |  |  | PDSCH Data |
| --- | --- | --- | --- | --- | --- |
|  | Subframe (Note 1) |  |  |  |  |
|  | 0 | 5 | 4, 9 | 1 - 3, 6 - 8 |  |
| 0 - 5 | 0 | 0 | 0 | 0 | Note 2 |
| NOTE 1: The allocation of any PDSCH with or without SIB1 applies only to the subframes not configured as PRS subframes. NOTE 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameter is used to scale the power of PDSCH. NOTE 3: If two or more transmit antennas with CRS are used in the test, the PDSCH part of OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the PDSCH part of OCNG is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. N/A: Not Applicable. |  |  |  |  |  |

## D.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS

Table D.1.8-1: OP.8 FDD: OCNG FDD Pattern 8

| Allocation | Relative power level [dB] |  |  |  | PDSCH Data |
| --- | --- | --- | --- | --- | --- |
|  | Subframe (Note 1) |  |  |  |  |
|  | 0 | 5 | 4,9 | (1-3, 6-8)Note4 |  |
| 0 - 12 | 0 | 0 | 0 | N/A | Note 2 |
| 37 - 49 | 0 | 0 | 0 | N/A |  |
| 0 - 49 | N/A | N/A | N/A | 0 |  |
| NOTE 1: The allocation of any PDSCH with or without SIB1 applies only to the subframes not configured as PRS subframes. NOTE 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameter is used to scale the power of PDSCH. NOTE 3: If two or more transmit antennas with CRS are used in the test, the PDSCH part of OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the PDSCH part of OCNG is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. NOTE 4: The subframe(s) configured as MBSFN ABS in a test shall not contain any PMCH data and shall contain CRS only in the first symbol of the first time slot. The subframe(s) configured as MBSFN ABS depend upon the MBSFN ABS pattern used in the test. N/A: Not Applicable. |  |  |  |  |  |

## D.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS

Table D.1.9-1: OP.9 FDD: OCNG FDD Pattern 9

| Allocation | Relative power level [dB] |  |  |  | PDSCH Data |
| --- | --- | --- | --- | --- | --- |
|  | Subframe (Note 1) |  |  |  |  |
|  | 0 | 5 | 4, 9 | (1-3, 6-8)Note4 |  |
| 0 - 49 | 0 | 0 | 0 | 0 | Note 2 |
| NOTE 1: The allocation of any PDSCH with or without SIB1 applies only to the subframes not configured as PRS subframes. NOTE 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameter is used to scale the power of PDSCH. NOTE 3: If two or more transmit antennas with CRS are used in the test, the PDSCH part of OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the PDSCH part of OCNG is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. NOTE 4: The subframe(s) configured as MBSFN ABS in a test shall not contain any PMCH data and shall contain CRS only in the first symbol of the first time slot. The subframe(s) configured as MBSFN ABS depend upon the MBSFN ABS pattern used in the test. N/A: Not Applicable. |  |  |  |  |  |

## D.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every SF (without MBSFN)

Table D.1.10-1: OP.10 FDD: OCNG FDD Pattern 10

| Allocation | Relative power level [dB] |  |  |  | PDSCH Data |
| --- | --- | --- | --- | --- | --- |
|  | Subframe (Note 1) |  |  |  |  |
|  | 0 | 5 | 4, 9 | 1 - 3, 6 - 8 |  |
| 0 - 12 | 0 | 0 | 0 | 0 | Note 2 |
| 37 - 49 | 0 | 0 | 0 | 0 |  |
| NOTE 1: The allocation of any PDSCH with or without SIB1 applies only to the subframes not configured as PRS subframes. NOTE 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameter is used to scale the power of PDSCH. NOTE 3: If two or more transmit antennas with CRS are used in the test, the PDSCH part of OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the PDSCH part of OCNG is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. N/A: Not Applicable. |  |  |  |  |  |

## D.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz

Table D.1.11-1: OP.11 FDD: OCNG FDD Pattern 11

| Allocation | Relative power level [dB] |  |  |  | PDSCH Data | PMCH Data |
| --- | --- | --- | --- | --- | --- | --- |
|  | Subframe |  |  |  |  |  |
|  | 0 | 5 | 4,9 | 1-3, 6-8 |  |  |
| 0 - 37 | 0 | 0 | 0 | N/A | Note 1 | N/A |
| 62 - 99 | 0 | 0 | 0 | N/A |  |  |
| 0-99 | N/A | N/A | N/A | Note 4 | N/A | Note 2 |
| NOTE 1: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameter is used to scale the power of PDSCH. NOTE 2: Each physical resource block (PRB) is assigned to MBSFN transmission. The data in each PRB shall be uncorrelated with data in other PRBs over the period of any measurement. The MBSFN data shall be QPSK modulated. PMCH symbols shall not contain cell-specific Reference Signals. PMCH subframes shall contain cell-specific Reference Signals only in the first symbol of the first time slot. The parameteris used to scale the power of PMCH. NOTE 3: If two or more transmit antennas with CRS are used in the test, the PDSCH part of OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the PDSCH part of OCNG is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. NOTE 4: 0dB for 1 transmit antenna with CRS, +3dB for 2 transmit antennas with CRS. N/A: Not Applicable. |  |  |  |  |  |  |

## D.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz

Table D.1.12-1: OP.12 FDD: OCNG FDD Pattern 12

| Allocation | Relative power level [dB] |  |  |  | PDSCH Data | PMCH Data |
| --- | --- | --- | --- | --- | --- | --- |
|  | Subframe |  |  |  |  |  |
|  | 0 | 5 | 4, 9 | 1 - 3, 6 - 8 |  |  |
| 0 - 99 | 0 | 0 | 0 | N/A | Note 1 | N/A |
| 0 - 99 | N/A | N/A | N/A | Note 4 | N/A | Note 2 |
| NOTE 1: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameter is used to scale the power of PDSCH. NOTE 2: Each physical resource block (PRB) is assigned to MBSFN transmission. The data in each PRB shall be uncorrelated with data in other PRBs over the period of any measurement. The MBSFN data shall be QPSK modulated. PMCH subframes shall contain cell-specific Reference Signals only in the first symbol of the first time slot. The parameteris used to scale the power of PMCH. NOTE 3: If two or more transmit antennas with CRS are used in the test, the PDSCH part of OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the PDSCH part of OCNG is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. NOTE 4: 0dB for 1 transmit antenna with CRS, +3dB for 2 transmit antennas with CRS. N/A: Not Applicable. |  |  |  |  |  |  |

## D.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)

Table D.1.13-1: OP.13 FDD: OCNG FDD Pattern 13

| Allocation | Relative power level [dB] |  |  |  | PDSCH Data |
| --- | --- | --- | --- | --- | --- |
|  | Subframe (Note 1) |  |  |  |  |
|  | 0 | 5 | 4,9 | 1-3, 6-8 |  |
| 0 - 37 | 0 | 0 | 0 | N/A | Note 2 |
| 62 - 99 | 0 | 0 | 0 | N/A |  |
| 0 - 99 | N/A | N/A | N/A | 0 |  |
| NOTE 1: The allocation of any PDSCH with or without SIB1 applies only to the subframes not configured as PRS subframes. NOTE 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameter is used to scale the power of PDSCH. NOTE 3: If two or more transmit antennas with CRS are used in the test, the PDSCH part of OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the PDSCH part of OCNG is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. N/A: Not Applicable. |  |  |  |  |  |

## D.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)

Table D.1.14-1: OP.14 FDD: OCNG FDD Pattern 14

| Allocation | Relative power level [dB] |  |  |  | PDSCH Data |
| --- | --- | --- | --- | --- | --- |
|  | Subframe (Note 1) |  |  |  |  |
|  | 0 | 5 | 4, 9 | 1 - 3, 6 - 8 |  |
| 0 - 99 | 0 | 0 | 0 | 0 | Note 2 |
| NOTE 1: The allocation of any PDSCH with or without SIB1 applies only to the subframes not configured as PRS subframes. NOTE 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameter is used to scale the power of PDSCH. NOTE 3: If two or more transmit antennas with CRS are used in the test, the PDSCH part of OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the PDSCH part of OCNG is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. N/A: Not Applicable. |  |  |  |  |  |

## D.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz

Table D.1.15-1: OP.15 FDD: OCNG FDD Pattern 15

| Allocation | Relative power level [dB] |  |  |  | PDSCH Data | PMCH Data |
| --- | --- | --- | --- | --- | --- | --- |
|  | Subframe |  |  |  |  |  |
|  | 0 | 5 | 4,9 | 1-3, 6-8 |  |  |
| 0 - 6 | 0 | 0 | 0 | N/A | Note 1 | N/A |
| 18 - 24 | 0 | 0 | 0 | N/A |  |  |
| 0-24 | N/A | N/A | N/A | Note 4 | N/A | Note 2 |
| NOTE 1: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameter is used to scale the power of PDSCH. NOTE 2: Each physical resource block (PRB) is assigned to MBSFN transmission. The data in each PRB shall be uncorrelated with data in other PRBs over the period of any measurement. The MBSFN data shall be QPSK modulated. PMCH symbols shall not contain cell-specific Reference Signals. PMCH subframes shall contain cell-specific Reference Signals only in the first symbol of the first time slot. The parameteris used to scale the power of PMCH. NOTE 3: If two or more transmit antennas with CRS are used in the test, the PDSCH part of OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the PDSCH part of OCNG is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. NOTE 4: 0dB for 1 transmit antenna with CRS, +3dB for 2 transmit antennas with CRS N/A: Not Applicable. |  |  |  |  |  |  |

## D.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz

Table D.1.16-1: OP.16 FDD: OCNG FDD Pattern 16

| Allocation | Relative power level [dB] |  |  |  | PDSCH Data | PMCH Data |
| --- | --- | --- | --- | --- | --- | --- |
|  | Subframe |  |  |  |  |  |
|  | 0 | 5 | 4, 9 | 1 - 3, 6 - 8 |  |  |
| 0 - 24 | 0 | 0 | 0 | N/A | Note 1 | N/A |
| 0 - 24 | N/A | N/A | N/A | Note 4 | N/A | Note 2 |
| NOTE 1: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameter is used to scale the power of PDSCH. NOTE 2: Each physical resource block (PRB) is assigned to MBSFN transmission. The data in each PRB shall be uncorrelated with data in other PRBs over the period of any measurement. The MBSFN data shall be QPSK modulated. PMCH subframes shall contain cell-specific Reference Signals only in the first symbol of the first time slot. The parameteris used to scale the power of PMCH. NOTE 3: If two or more transmit antennas with CRS are used in the test, the PDSCH part of OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the PDSCH part of OCNG is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. NOTE 4: 0dB for 1 transmit antenna with CRS, +3dB for 2 transmit antennas with CRS N/A: Not Applicable. |  |  |  |  |  |  |

## D.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)

Table D.1.17-1: OP.17 FDD: OCNG FDD Pattern 17

| Allocation | Relative power level [dB] |  |  |  | PDSCH Data |
| --- | --- | --- | --- | --- | --- |
|  | Subframe (Note 1) |  |  |  |  |
|  | 0 | 5 | 4, 9 | 1 - 3, 6 - 8 |  |
| 0 - 37 | 0 | 0 | 0 | 0 | Note 2 |
| 62 - 99 | 0 | 0 | 0 | 0 |  |
| NOTE 1: The allocation of any PDSCH with or without SIB1 applies only to the subframes not configured as PRS subframes. NOTE 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameter is used to scale the power of PDSCH. NOTE 3: If two or more transmit antennas with CRS are used in the test, the PDSCH part of OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the PDSCH part of OCNG is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. N/A: Not Applicable. |  |  |  |  |  |

## D.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)

Table D.1.18-1: OP.18 FDD: OCNG FDD Pattern 18

| Allocation | Relative power level [dB] |  |  |  | PDSCH Data |
| --- | --- | --- | --- | --- | --- |
|  | Subframe (Note 1) |  |  |  |  |
|  | 0 | 5 | 4,9 | 1-3, 6-8 |  |
| 0 - 6 | 0 | 0 | 0 | N/A | Note 2 |
| 18 - 24 | 0 | 0 | 0 | N/A |  |
| 0 - 24 | N/A | N/A | N/A | 0 |  |
| NOTE 1: PDSCH allocation applies only to subframes not configured as PRS subframes. NOTE 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameter is used to scale the power of PDSCH. NOTE 3: If two or more transmit antennas with CRS are used in the test, the PDSCH part of OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the PDSCH part of OCNG is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. N/A: Not Applicable. |  |  |  |  |  |

## D.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)

Table D.1.19-1: OP.19 FDD: OCNG FDD Pattern 19

| Allocation | Relative power level [dB] |  |  |  | PDSCH Data |
| --- | --- | --- | --- | --- | --- |
|  | Subframe (Note 1) |  |  |  |  |
|  | 0 | 5 | 4, 9 | 1 - 3, 6 - 8 |  |
| 0 - 24 | 0 | 0 | 0 | 0 | Note 2 |
| NOTE 1: PDSCH allocation applies only to subframes not configured as PRS subframes. NOTE 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameter is used to scale the power of PDSCH. NOTE 3: If two or more transmit antennas with CRS are used in the test, the PDSCH part of OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the PDSCH part of OCNG is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. N/A: Not Applicable. |  |  |  |  |  |

## D.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)

Table D.1.20-1: OP.20 FDD: OCNG FDD Pattern 20

| Allocation | Relative power level [dB] |  |  |  | PDSCH Data |
| --- | --- | --- | --- | --- | --- |
|  | Subframe (Note 1) |  |  |  |  |
|  | 0 | 5 | 4, 9 | 1 - 3, 6 - 8 |  |
| 0 - 6 | 0 | 0 | 0 | 0 | Note 2 |
| 18 - 24 | 0 | 0 | 0 | 0 |  |
| NOTE 1: PDSCH allocation applies only to subframes not configured as PRS subframes. NOTE 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameter is used to scale the power of PDSCH. NOTE 3: If two or more transmit antennas with CRS are used in the test, the PDSCH part of OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the PDSCH part of OCNG is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. N/A: Not Applicable. |  |  |  |  |  |

## D.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)

Table D.1.21-1: OP.21 FDD: OCNG FDD Pattern 21

| OCNG Pattern Name | Bandwidth (MHz) | Allocation | Relative power level [dB] |  |  |  | PDSCH Data |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Subframe |  |  |  |  |
|  |  |  | 0 | 5 | 4, 9 | 1 - 3, 6 - 8 |  |
| OP.21 FDD | 10 | 0 - 49Note 1,2 | 0 | 0 | 0 | 0 | Note 3 |
| NOTE 1: The OCNG pattern is used only for a serving cell of the UE under test. NOTE 2: The OCNG allocation applied to all downlink physical resource blocks (PRBs) except the REs that are allocated for the PDSCH for the UE under test. NOTE 3: OCNG PRBs are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameter is used to scale the power of PDSCH. NOTE 4: If two or more transmit antennas with CRS are used in the test, the PDSCH part of OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the PDSCH part of OCNG is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. |  |  |  |  |  |  |  |

# D.2	OCNG Patterns for TDD

## D.2.0

The following OCNG patterns are used for modelling allocations to virtual UEs (which are not under test). The OCNG pattern for each sub frame specifies the allocations that shall be filled with OCNG, and furthermore, the relative power level of each such allocation.

In each test case the OCNG is expressed by parameters OCNG_RA and OCNG_RB which together with a relative power level () specifies the PDSCH EPRE-to-RS EPRE ratios in OFDM symbols with and without reference symbols, respectively. The relative power, which is used for modelling boosting per virtual UE allocation, is expressed by:

where  denotes the relative power level of the i:th virtual UE. The parameter settings of OCNG_RA, OCNG_RB, and the set of relative power levels are chosen such that when also taking allocations to the UE under test into account, as given by a PDSCH reference channel, a transmitted power spectral density that is constant on an OFDM symbol basis is targeted.

Moreover the OCNG pattern is accompanied by a PCFICH/PDCCH/PHICH reference channel which specifies the control region. The number of PDCCH OFDM symbols in the non-MBSFN subframes is the same as specified in the RMC used in the test. The number of PDCCH OFDM symbols in the MBSFN subframes is the maximal allowed according to 3GPP TS 36.213 [8]. For any aggregation and PHICH allocation, the PDCCH and any unused PHICH groups are padded with resource element groups with a power level given respectively by PDCCH_RA/RB and PHICH_RA/RB as specified in the test case such that a total power spectral density in the control region that is constant on an OFDM symbol basis is targeted.

For subframes configured as PRS subframes the PDSCH allocation defined in the OCNG pattern does not apply.

For subframes configured as ABS subframes the PDSCH and PMCH allocation defined in the OCNG pattern does not apply.

The system information is scheduled in the allocations reserved for the OCNG patterns, in the subframes not configured for MBSFN. For this purpose the number of the RB-s allocated with PDSCH defined in the OCNG pattern can be reduced as necessary.

## D.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz

Table D.2.1-1: OP.1 TDD: OCNG TDD Pattern 1 for 5ms downlink-to-uplink switch-point periodicity

| Allocation | Relative power level [dB] |  |  |  | PDSCH Data |
| --- | --- | --- | --- | --- | --- |
|  | Subframe (Note 1) |  |  |  |  |
|  | 0 | 5 | 3, 4, 8, 9 and 6 (as normal subframe) Note 3 | 1 and 6 (as special subframe) Note 3 |  |
| 0 - 12 | 0 | 0 | 0 | 0 | Note 2 |
| 37 - 49 | 0 | 0 | 0 | 0 |  |
| NOTE 1: The allocation of any PDSCH with or without SIB1 applies only to the subframes not configured as PRS subframes. NOTE 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameteris used to scale the power of PDSCH. NOTE 3: Subframes available for DL transmission depends on the Uplink-Downlink configuration defined in Table 4.2-2 in 3GPP TS 36.211 [9]. The control region consists of PCFICH, PHICH and PDCCH. Number of OFDM symbols belonging to the control region may vary between subframes. NOTE 4: If two or more transmit antennas with CRS are used in the test, the OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. NOTE 5: If SPDCCH is to be transmitted on a physical resource block(PRB), this takes priority over OCNG transmission. |  |  |  |  |  |

## D.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz

Table D.2.2-1: OP.2 TDD: OCNG TDD Pattern 2 for 5ms downlink-to-uplink switch-point periodicity

| Allocation | Relative power level [dB] |  |  |  | PDSCH Data |
| --- | --- | --- | --- | --- | --- |
|  | Subframe (Note 1) |  |  |  |  |
|  | 0 | 5 | 3 , 4, 8, 9 and 6 (as normal subframe) Note 3 | 1 and 6 (as special subframe) Note 3 |  |
| 0 - 49 | 0 | 0 | 0 | 0 | Note 2 |
| NOTE 1: The allocation of any PDSCH with or without SIB1 applies only to the subframes not configured as PRS subframes. NOTE 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameteris used to scale the power of PDSCH. NOTE 3: Subframes available for DL transmission depends on the Uplink-Downlink configuration in Table 4.2-2 in 3GPP TS 36.211 [9]. The control region consists of PCFICH, PHICH and PDCCH. Number of OFDM symbols belonging to the control region may vary between subframes. NOTE 4: If two or more transmit antennas with CRS are used in the test, the OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. |  |  |  |  |  |

## D.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz

Table D.2.3-1: OP.3 TDD: OCNG TDD Pattern 3 for 5 ms downlink-to-uplink switch-point periodicity

| Allocation | Relative power level [dB] |  |  |  | PDSCH Data |
| --- | --- | --- | --- | --- | --- |
|  | Subframe (Note 1) |  |  |  |  |
|  | 0 | 5 | 3 , 4, 8, 9 and 6 (as normal subframe) Note 3 | 1 and 6 (as special subframe) Note 3 |  |
| 0 - 1 | 0 | 0 | 0 | 0 | Note 2 |
| 4 - 5 | 0 | 0 | 0 | 0 |  |
| NOTE 1: The allocation of any PDSCH with or without SIB1 applies only to the subframes not configured as PRS subframes. NOTE 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameteris used to scale the power of PDSCH. NOTE 3: Subframes available for DL transmission depends on the Uplink-Downlink configuration defined in Table 4.2-2 in 3GPP TS 36.211 [9]. NOTE 4: If two or more transmit antennas with CRS are used in the test, the OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. |  |  |  |  |  |

## D.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz

Table D.2.4-1: OP.4 TDD: OCNG TDD Pattern 4 for 5 ms downlink-to-uplink switch-point periodicity

| Allocation | Relative power level [dB] |  |  |  | PDSCH Data |
| --- | --- | --- | --- | --- | --- |
|  | Subframe (Note 1) |  |  |  |  |
|  | 0 | 5 | 3 , 4, 8, 9 and 6 (as normal subframe) Note 3 | 1 and 6 (as special subframe) Note 3 |  |
| 0 - 5 | 0 | 0 | 0 | 0 | Note 2 |
| NOTE 1: The allocation of any PDSCH with or without SIB1 applies only to the subframes not configured as PRS subframes. NOTE 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameteris used to scale the power of PDSCH. NOTE 3: Subframes available for DL transmission depends on the Uplink-Downlink configuration in Table 4.2-2 in 3GPP TS 36.211 [9]. NOTE 4: If two or more transmit antennas with CRS are used in the test, the OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. |  |  |  |  |  |

## D.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS

Table D.2.5-1: OP.5 TDD: OCNG TDD Pattern 5 for 5ms downlink-to-uplink switch-point periodicity

| Allocation | Relative power level [dB] |  |  |  | PDSCH Data |
| --- | --- | --- | --- | --- | --- |
|  | Subframe (Note 1) |  |  |  |  |
|  | 0 | 5 | 3 , 4, 8, 9 and 6 (as normal subframe) Note 3 | 1 and 6 (as special subframe) Note 3 |  |
| 0 - 12 | 0 | 0 | 0 | Table D.2.5-2 | Note 2 |
| 37 - 49 | 0 | 0 | 0 |  |  |
| NOTE 1: The allocation of any PDSCH with or without SIB1 applies only to the subframes not configured as PRS subframes. NOTE 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated The parameteris used to scale the power of PDSCH. NOTE 3: Subframes available for DL transmission depends on the Uplink-Downlink configuration defined in Table 4.2-2 in 3GPP TS 36.211 [16]. Any of the subframes 3, 4, 8 and 9 configured as MBSFN ABS in a test shall not contain any PMCH data and shall contain CRS only in the first symbol of the first time slot. The subframe(s) configured as MBSFN ABS depend upon the MBSFN ABS pattern used in the test. NOTE 4: If two or more transmit antennas with CRS are used in the test, the OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. |  |  |  |  |  |

Table D.2.5-2: OP.5 TDD: OCNG TDD Pattern 5 for special subframe configuration with 5ms downlink-to-uplink switch-point periodicity

| Allocation | CP length | Relative power level [dB] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Special subframe configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 0 |  | 1 |  | 2 |  | 3 |  | 4 |  | 5 |  | 6 |  | 7 |  | 8 |  |
|  |  | Control region OFDM symbols |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 |
| 0 - 12 | N | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |
| 37 - 49 | N | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |
| NOTE: Special subframe configurations are defined in Table 4.2-1 in 3GPP TS 36.211 [16]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## D.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS

Table D.2.6-1: OP.6 TDD: OCNG TDD Pattern 6 for 5ms downlink-to-uplink switch-point periodicity

| Allocation | Relative power level [dB] |  |  |  | PDSCH Data |
| --- | --- | --- | --- | --- | --- |
|  | Subframe (Note 1) |  |  |  |  |
|  | 0 | 5 | 3 , 4, 8, 9 and 6 (as normal subframe)Note 3 | 1 and 6 (as special subframe) Note 3 |  |
| 0 - 49 | 0 | 0 | 0 | 0 | Note 2 |
| NOTE 1: The allocation of any PDSCH with or without SIB1 applies only to the subframes not configured as PRS subframes. NOTE 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameteris used to scale the power of PDSCH. NOTE 3: Subframes available for DL transmission depends on the Uplink-Downlink configuration in Table 4.2-2 in 3GPP TS 36.211 [16]. Any of the subframes 3, 4, 8 and 9 configured as MBSFN ABS in a test shall not contain any PMCH data and shall contain CRS only in the first symbol of the first time slot. The subframe(s) configured as MBSFN ABS depend upon the MBSFN ABS pattern used in the test. NOTE 4: If two or more transmit antennas with CRS are used in the test, the OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. |  |  |  |  |  |

## D.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz

Table D.2.7-1: OP.7 TDD: OCNG TDD Pattern 7 for 5ms downlink-to-uplink switch-point periodicity

| Allocation | Relative power level [dB] |  |  |  | PDSCH Data |
| --- | --- | --- | --- | --- | --- |
|  | Subframe (Note 1) |  |  |  |  |
|  | 0 | 5 | 3 , 4, 8, 9 and 6 (as normal subframe) Note 3 | 1 and 6 (as special subframe) Note 3 |  |
| 0 - 37 | 0 | 0 | 0 | Table D.2.7-2 | Note 2 |
| 62 - 99 | 0 | 0 | 0 |  |  |
| NOTE 1: The allocation of any PDSCH with or without SIB1 applies only to the subframes not configured as PRS subframes. NOTE 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated The parameteris used to scale the power of PDSCH. NOTE 3: Subframes available for DL transmission depends on the Uplink-Downlink configuration defined in Table 4.2-2 in 3GPP TS 36.211 [16]. NOTE 4: If two or more transmit antennas with CRS are used in the test, the OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. |  |  |  |  |  |

Table D.2.7-2: OP.7 TDD: OCNG TDD Pattern 7 for special subframe configuration with 5ms downlink-to-uplink switch-point periodicity

| Allocation | CP length | Relative power level [dB] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Special subframe configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 0 |  | 1 |  | 2 |  | 3 |  | 4 |  | 5 |  | 6 |  | 7 |  | 8 |  |
|  |  | Control region OFDM symbols |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 |
| 0 - 37 | N | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |
| 62 - 99 | N | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |
| NOTE: Special subframe configurations are defined in Table 4.2-1 in 3GPP TS 36.211 [16]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## D.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz

Table D.2.8-1: OP.8 TDD: OCNG TDD Pattern 8 for 5ms downlink-to-uplink switch-point periodicity

| Allocation | Relative power level [dB] |  |  |  | PDSCH Data |
| --- | --- | --- | --- | --- | --- |
|  | Subframe (Note 1) |  |  |  |  |
|  | 0 | 5 | 3 , 4, 8, 9 and 6 (as normal subframe)Note 3 | 1 and 6 (as special subframe) Note 3 |  |
| 0 - 99 | 0 | 0 | 0 | 0 | Note 2 |
| NOTE 1: The allocation of any PDSCH with or without SIB1 applies only to the subframes not configured as PRS subframes. NOTE 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameteris used to scale the power of PDSCH. NOTE 3: Subframes available for DL transmission depends on the Uplink-Downlink configuration in Table 4.2-2 in 3GPP TS 36.211 [16]. NOTE 4: If two or more transmit antennas with CRS are used in the test, the OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. |  |  |  |  |  |

## D.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz

Table D.2.9-1: OP.9 TDD: OCNG TDD Pattern 9 for 5ms downlink-to-uplink switch-point periodicity

| Allocation | Relative power level [dB] |  |  |  | PDSCH Data |
| --- | --- | --- | --- | --- | --- |
|  | Subframe (Note 1) |  |  |  |  |
|  | 0 | 5 | 3 , 4, 8, 9 and 6 (as normal subframe) Note 3 | 1 and 6 (as special subframe) Note 3 |  |
| 0 - 6 | 0 | 0 | 0 | Table A.3.2.1.7-2 | Note 2 |
| 18 - 24 | 0 | 0 | 0 |  |  |
| NOTE 1: PDSCH allocation applies only to subframes not configured as PRS subframes. NOTE 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated The parameteris used to scale the power of PDSCH. NOTE 3: Subframes available for DL transmission depends on the Uplink-Downlink configuration defined in Table 4.2-2 in 3GPP TS 36.211 [16]. NOTE 4: If two or more transmit antennas with CRS are used in the test, the OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. |  |  |  |  |  |

Table D.2.9-2: OP.9 TDD: OCNG TDD Pattern 9 for special subframe configuration with 5ms downlink-to-uplink switch-point periodicity

| Allocation | CP length | Relative power level [dB] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Special subframe configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 0 |  | 1 |  | 2 |  | 3 |  | 4 |  | 5 |  | 6 |  | 7 |  | 8 |  |
|  |  | Control region OFDM symbols |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 |
| 0 - 6 | N | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |
| 18 - 24 | N | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |
| NOTE: Special subframe configurations are defined in Table 4.2-1 in 3GPP TS 36.211 [16]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## D.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz

Table D.2.10-1: OP.10 TDD: OCNG TDD Pattern 10 for 5ms downlink-to-uplink switch-point periodicity

| Allocation | Relative power level [dB] |  |  |  | PDSCH Data |
| --- | --- | --- | --- | --- | --- |
|  | Subframe (Note 1) |  |  |  |  |
|  | 0 | 5 | 3 , 4, 8, 9 and 6 (as normal subframe)Note 3 | 1 and 6 (as special subframe) Note 3 |  |
| 0 - 24 | 0 | 0 | 0 | 0 | Note 2 |
| NOTE 1: PDSCH allocation applies only to subframes not configured as PRS subframes. NOTE 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameteris used to scale the power of PDSCH. NOTE 3: Subframes available for DL transmission depends on the Uplink-Downlink configuration in Table 4.2-2 in 3GPP TS 36.211 [16]. NOTE 4: If two or more transmit antennas with CRS are used in the test, the OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. |  |  |  |  |  |

## D.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)

Table D.2.11-1: OP.11 TDD: OCNG TDD Pattern 11

| OCNG Pattern Name | Bandwidth (MHz) | Allocation | Relative power level [dB] |  |  |  | PDSCH Data |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Subframe |  |  |  |  |
|  |  |  | 0 | 5 | 3 , 4, 8, 9 and 6 (as normal subframe) Note 5 | 1 and 6 (as special subframe) Note 5 |  |
| OP.11 TDD | 10 | 0 - 49Note 1,2 | 0 | 0 | 0 | 0 | Note 3 |
| NOTE 1: The OCNG pattern is used only for a serving cell of the UE under test. NOTE 2: The OCNG allocation applied to all downlink physical resource blocks (PRBs) except the REs that are allocated for the PDSCH for the UE under test. NOTE 3: OCNG PRBs are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameter is used to scale the power of PDSCH. NOTE 4: If two or more transmit antennas with CRS are used in the test, the PDSCH part of OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the PDSCH part of OCNG is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. NOTE 5: Subframes available for DL transmission depends on the Uplink-Downlink configuration in Table 4.2-2 in 3GPP TS 36.211 [16] |  |  |  |  |  |  |  |

# D.3	OCNG Patterns for Narrowband IoT

## D.3.0

The following Narrowband OCNG patterns (NOCNG) are used for modelling allocations to UEs not under test in a Narrowband IoT cell. Depending on scenario, allocations may be for UEs of category NB1only, or for UEs of category NB1 as well as of other categories. The former is applicable to guard-band and stand-alone deployments of Narrowband IoT, whereas the latter is applicable to in-band deployment. In order to allow different power levels for the LTE cell and the Narrowband IoT cell, a distinction is made between OCNG and NOCNG where the latter is used for category NB1 UEs and the former is used for other UE categories.

OCNG in the LTE cell is expressed by parameters OCNG_RA and OCNG_RB which together with a relative power level () specifies the PDSCH-to-RS EPRE ratio in OFDM symbols with and without LTE cell-specific reference symbols, respectively. The relative power, which is used for modelling boosting per virtual LTE UE allocation, is expressed by:

where  denotes the relative power level of the i:th virtual LTE UE.

Moreover in each test case NOCNG is expressed by parameters NOCNG_RA and NOCNG_RB which together with a relative power level () specifies the <channel>-to-RS EPRE ratio in OFDM symbols with and without Narrowband reference symbols (NB-RS), respectively. The relative power, which is used for modelling boosting per virtual UE category NB1 allocation, is expressed by:

where  denotes the relative power level of the k:th virtual NB-IoT UE, and channel may be either of NPDCCH and NPDSCH.

The parameter settings of OCNG_RA, OCNG_RB, NOCNG_RA, NOCNG_RB and the set of relative power levels are chosen such that when also taking allocations to the UE category NB1 under test into account, as given by a NPDCCH and NPDSCH reference channels, a transmitted power spectral density that is constant on an OFDM symbol basis is targeted.

## D.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell

Table D.3.1-1: NOP.1 FDD: OCNG FDD Pattern 1

| Allocation | Relative power level [dB] |  |  | PDSCH Data | NPDCCH and NPDSCH Data |
| --- | --- | --- | --- | --- | --- |
|  | Subframe |  |  |  |  |
|  | 0, 4 | 5, 9 | 1-3, 6-8 |  |  |
| 0 - ( nCell[1]RB -1) (Note 5) | 0 | 0 | 0 | Note 1 | N/A |
| nCell[1]RB (Note 5) | 0 (Note 3) | 0 (Note 3) | N/A | N/A | Note 2 |
|  | N/A | N/A | 0 | Note 1 | N/A |
| ( nCell[1]RB +1 ) - ( nCell[…]RB -1) (Note 5) | 0 | 0 | 0 | Note 1 | N/A |
| nCell[…]RB (Note 5) | 0 (Note 3) | 0 (Note 3) | N/A | N/A | Note 2 |
|  | N/A | N/A | 0 | Note 1 | N/A |
| ( nCell[...]RB +1 ) - ( nCell[N]RB -1) (Note 5) | 0 | 0 | 0 | Note 1 | N/A |
| nCell[N]RB (Note 5) | 0 (Note 3) | 0 (Note 3) | N/A | N/A | Note 2 |
|  | N/A | N/A | 0 | Note 1 | N/A |
| ( nCell[N]RB+1 ) - 49 (Note 5) | 0 | 0 | 0 | Note 1 | N/A |
| NOTE 1: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated QPSK modulated pseudo random data. The parameter is used to scale the power of PDSCH. If two or more transmit antennas with CRS are used in the test, the PDSCH part of OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the PDSCH part of OCNG is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. NOTE 2: This physical resource block is assigned to a virtual UE for transmission of NPDCCH or NPDSCH; the data transmitted over the NOCNG NPDCCH or NPDSCH shall be uncorrelated QPSK modulated pseudo random data. The parameter is used to scale the power of NPDCCH and NPDSCH. If two transmit antennas with NBRS are used in the test, the NPDCCH and NPDSCH parts of NOCNG shall be transmitted to the virtual users by both transmit antennas with NBRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the NPDCCH and NPDSCH parts of NOCNG is equal between both the transmit antennas with NBRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. NOTE 3: Value of is applicable to PRBs not used for transmission of NPSS, NSSS, and NPBCH in anchor cell. NOTE 4: SI transmissions, and NPDCCH and NPDSCH for the UE under test, have precedence over OCNG and NOCNG. NOTE 5: nCell[k]RB is the index of the RB used for allocation of the NB-IoT cell k with k = {1, .., N} with N the total number of the NB-IoT cells belonging to the same LTE donor cell as specified in the individual tests. N/A: Not Applicable |  |  |  |  |  |

## D.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell

Table D.3.2-1: NOP.2 FDD: OCNG FDD Pattern 2

| Allocation | Relative power level [dB] |  |  | PDSCH Data | NPDCCH and NPDSCH Data |
| --- | --- | --- | --- | --- | --- |
|  | Subframe |  |  |  |  |
|  | 0, 4 | 5, 9 | 1-3, 6-8 |  |  |
| 0-49 | 0 | 0 | 0 | Note 1 | N/A |
| 50 | 0 | 0 (Note 3) | 0 | N/A | Note 2 |
| NOTE 1: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated QPSK modulated pseudo random data. The parameter is used to scale the power of PDSCH. If two or more transmit antennas with CRS are used in the test, the PDSCH part of OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the PDSCH part of OCNG is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. NOTE 2: This physical resource block is assigned to a virtual UE for transmission of NPDCCH or NPDSCH; the data transmitted over the NOCNG NPDCCH or NPDSCH shall be uncorrelated QPSK modulated pseudo random data. The parameter is used to scale the power of NPDCCH and NPDSCH. If two transmit antennas with NBRS are used in the test, the NPDCCH and NPDSCH parts of NOCNG shall be transmitted to the virtual users by both transmit antennas with NBRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the NPDCCH and NPDSCH parts of NOCNG is equal between both the transmit antennas with NBRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. NOTE 3: Value of is applicable to PRBs not used for transmission of NPSS, NSSS and PBCH in anchor cell (= 50). NOTE 4: SI transmissions, and NPDCCH and NPDSCH for the UE under test, have precedence over OCNG and NOCNG. N/A: Not Applicable |  |  |  |  |  |

## D.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT

Table D.3.3-1: NOP.3 FDD: OCNG FDD Pattern 3

| Allocation | Relative power level [dB] |  |  | NPDCCH and NPDSCH Data |
| --- | --- | --- | --- | --- |
|  | Subframe |  |  |  |
|  | 0, 4 | 5, 9 | 1-3, 6-8 |  |
| 0 | 0 (Note 2) | 0 (Note 2) | 0 | Note 1 |
| NOTE 1: This physical resource block is assigned to a virtual UE for transmission of NPDCCH or NPDSCH; the data transmitted over the NOCNG NPDCCH or NPDSCH shall be uncorrelated QPSK modulated pseudo random data. The parameter is used to scale the power of NPDCCH and NPDSCH. If two transmit antennas with NBRS are used in the test, the NPDCCH and NPDSCH parts of NOCNG shall be transmitted to the virtual users by both transmit antennas with NBRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the NPDCCH and NPDSCH parts of NOCNG is equal between both the transmit antennas with NBRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. NOTE 2: Value of is applicable to PRBs not used for transmission of NPSS, NSSS and PBCH in anchor cell (= 0). NOTE 3: SI transmissions, and NPDCCH and NPDSCH for the UE under test, have precedence over NOCNG. |  |  |  |  |

## D.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell

Table D.3.4-1: NOP.4 FDD: OCNG FDD Pattern 4

| Allocation | Relative power level [dB] |  |  | PDSCH Data | NPDCCH and NPDSCH Data |
| --- | --- | --- | --- | --- | --- |
|  | Subframe |  |  |  |  |
|  | 0, 4 | 5, 9 | 1-3, 6-8 |  |  |
| 0 - ( nCell[1]RB -1) (Note 5) | 0 | 0 | 0 | Note 1 | N/A |
| nCell[1]RB (Note 5) | 0 (Note 3) | 0 (Note 3) | N/A | N/A | Note 2 |
|  | N/A | N/A | 0 | Note 1 | N/A |
| ( nCell[1]RB +1 ) - ( nCell[…]RB -1) (Note 5) | 0 | 0 | 0 | Note 1 | N/A |
| nCell[…]RB (Note 5) | 0 (Note 3) | 0 (Note 3) | N/A | N/A | Note 2 |
|  | N/A | N/A | 0 | Note 1 | N/A |
| ( nCell[...]RB +1 ) - ( nCell[N]RB -1) (Note 5) | 0 | 0 | 0 | Note 1 | N/A |
| nCell[N]RB (Note 5) | 0 (Note 3) | 0 (Note 3) | N/A | N/A | Note 2 |
|  | N/A | N/A | 0 | Note 1 | N/A |
| ( nCell[N]RB+1 ) - 24 (Note 5) | 0 | 0 | 0 | Note 1 | N/A |
| NOTE 1: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated QPSK modulated pseudo random data. The parameter is used to scale the power of PDSCH. If two or more transmit antennas with CRS are used in the test, the PDSCH part of OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the PDSCH part of OCNG is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. NOTE 2: This physical resource block is assigned to a virtual UE for transmission of NPDCCH or NPDSCH; the data transmitted over the NOCNG NPDCCH or NPDSCH shall be uncorrelated QPSK modulated pseudo random data. The parameter is used to scale the power of NPDCCH and NPDSCH. If two transmit antennas with NBRS are used in the test, the NPDCCH and NPDSCH parts of NOCNG shall be transmitted to the virtual users by both transmit antennas with NBRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the NPDCCH and NPDSCH parts of NOCNG is equal between both the transmit antennas with NBRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. NOTE 3: Value of is applicable to PRBs not used for transmission of NPSS, NSSS, and NPBCH in anchor cells. NOTE 4: SI transmissions, and NPDCCH and NPDSCH for the UE under test, have precedence over OCNG and NOCNG. NOTE 5: nCell[k]RB is the index of the RB used for allocation of the NB-IoT cell k with k = {1, .., N} with N the total number of the NB-IoT cells belonging to the same LTE donor cell as specified in the individual tests. N/A: Not Applicable |  |  |  |  |  |

## D.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell

Table D.3.5-1: NOP.5 FDD: OCNG FDD Pattern 5

| Allocation | Relative power level [dB] |  |  | PDSCH Data | NPDCCH and NPDSCH Data |
| --- | --- | --- | --- | --- | --- |
|  | Subframe |  |  |  |  |
|  | 0, 4 | 5, 9 | 1-3, 6-8 |  |  |
| 0 - 24 | 0 | 0 | 0 | Note 1 | N/A |
| 25 | 0 (Note 3) | 0 (Note 3) | 0 | N/A | Note 2 |
| NOTE 1: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated QPSK modulated pseudo random data. The parameter is used to scale the power of PDSCH. If two or more transmit antennas with CRS are used in the test, the PDSCH part of OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the PDSCH part of OCNG is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. NOTE 2: This physical resource block is assigned to a virtual UE for transmission of NPDCCH or NPDSCH; the data transmitted over the NOCNG NPDCCH or NPDSCH shall be uncorrelated QPSK modulated pseudo random data. The parameter is used to scale the power of NPDCCH and NPDSCH. If two transmit antennas with NBRS are used in the test, the NPDCCH and NPDSCH parts of NOCNG shall be transmitted to the virtual users by both transmit antennas with NBRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the NPDCCH and NPDSCH parts of NOCNG is equal between both the transmit antennas with NBRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. NOTE 3: Value of is applicable to PRBs not used for transmission of NPSS, NSSS and PBCH in anchor cell (= 25). NOTE 4: SI transmissions, and NPDCCH and NPDSCH for the UE under test, have precedence over OCNG and NOCNG. N/A: Not Applicable |  |  |  |  |  |

## D.3.6	Narrowband IoT OCNG TDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell

Table D.3.6-1: NOP.1 TDD: OCNG TDD Pattern 1 for uplink-downlink configurations 1 and 2

| Allocation | Relative power level [dB] |  |  | PDSCH Data | NPDCCH and NPDSCH Data |
| --- | --- | --- | --- | --- | --- |
|  | Subframe |  |  |  |  |
|  | 0, 5, 9 | 1, 6Note 7 | 3, 4, 8Note 4 |  |  |
| 0 - ( nCell[1]RB -1) (Note 5) | 0 | 0 | 0 | Note 1 | N/A |
| nCell[1]RB (Note 5) | 0 (Note 3) | 0 | N/A | N/A | Note 2 |
|  | N/A | N/A | 0 | Note 1 | N/A |
| ( nCell[1]RB +1 ) - ( nCell[…]RB -1) (Note 5) | 0 | 0 | 0 | Note 1 | N/A |
| nCell[…]RB (Note 5) | 0 (Note 3) | 0 | N/A | N/A | Note 2 |
|  | N/A | N/A | 0 | Note 1 | N/A |
| ( nCell[...]RB +1 ) - ( nCell[N]RB -1) (Note 5) | 0 | 0 | 0 | Note 1 | N/A |
| nCell[N]RB (Note 5) | 0 (Note 3) | 0 | N/A | N/A | Note 2 |
|  | N/A | N/A | 0 | Note 1 | N/A |
| ( nCell[N]RB+1 ) - 49 (Note 5) | 0 | 0 | 0 | Note 1 | N/A |
| NOTE 1: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated QPSK modulated pseudo random data. The parameter is used to scale the power of PDSCH. If two or more transmit antennas with CRS are used in the test, the PDSCH part of OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the PDSCH part of OCNG is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. NOTE 2: This physical resource block is assigned to a virtual UE for transmission of NPDCCH or NPDSCH; the data transmitted over the NOCNG NPDCCH or NPDSCH shall be uncorrelated QPSK modulated pseudo random data. The parameter is used to scale the power of NPDCCH and NPDSCH. If two transmit antennas with NBRS are used in the test, the NPDCCH and NPDSCH parts of NOCNG shall be transmitted to the virtual users by both transmit antennas with NBRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the NPDCCH and NPDSCH parts of NOCNG is equal between both the transmit antennas with NBRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. NOTE 3: Value of is applicable to PRBs not used for transmission of NPSS, NSSS, and NPBCH in anchor cells. NOTE 4: Subframes available for DL transmission depends on the Uplink-Downlink configuration in Table 4.2-2 in 3GPP TS 36.211 [16]. NOTE 5: nCell[k]RB is the index of the RB used for allocation of the NB-IoT cell k with k = {1, .., N} with N the total number of the NB-IoT cells belonging to the same E-UTRA donor cell as specified in the individual tests. NOTE 6: SI transmissions, and NPDCCH and NPDSCH for the UE under test, have precedence over OCNG and NOCNG. NOTE 7: Whether NPDSCH transmissions are carried out in DwPTS depends on the special subframe configuration in use. N/A: Not Applicable |  |  |  |  |  |

## D.3.7	Narrowband IoT OCNG TDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell

Table D.3.7-1: NOP.2 TDD: OCNG TDD Pattern 2 for uplink-downlink configurations 1 and 2

| Allocation | Relative power level [dB] |  |  | PDSCH Data | NPDCCH and NPDSCH Data |
| --- | --- | --- | --- | --- | --- |
|  | Subframe |  |  |  |  |
|  | 0, 5, 9 | 1, 6Note 6 | 3, 4, 8Note 4 |  |  |
| 0-49 | 0 | 0 | 0 | Note 1 | N/A |
| 50 | 0 (Note 3) | 0 | 0 | N/A | Note 2 |
| NOTE 1: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated QPSK modulated pseudo random data. The parameter is used to scale the power of PDSCH. If two or more transmit antennas with CRS are used in the test, the PDSCH part of OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the PDSCH part of OCNG is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. NOTE 2: This physical resource block is assigned to a virtual UE for transmission of NPDCCH or NPDSCH; the data transmitted over the NOCNG NPDCCH or NPDSCH shall be uncorrelated QPSK modulated pseudo random data. The parameter is used to scale the power of NPDCCH and NPDSCH. If two transmit antennas with NBRS are used in the test, the NPDCCH and NPDSCH parts of NOCNG shall be transmitted to the virtual users by both transmit antennas with NBRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the NPDCCH and NPDSCH parts of NOCNG is equal between both the transmit antennas with NBRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. NOTE 3: Value of is applicable to PRBs not used for transmission of NPSS, NSSS and NPBCH in anchor cell (= 50). NOTE 4: Subframes available for DL transmission depends on the Uplink-Downlink configuration in Table 4.2-2 in 3GPP TS 36.211 [16]. NOTE 5: SI transmissions, and NPDCCH and NPDSCH for the UE under test, have precedence over OCNG and NOCNG. NOTE 6: Whether NPDSCH transmissions are carried out in DwPTS depends on the special subframe configuration in use. N/A: Not Applicable |  |  |  |  |  |

## D.3.8	Narrowband IoT OCNG TDD pattern 3: standalone NB-IoT

Table D.3.8-1: NOP.3 TDD: OCNG TDD Pattern 3 for uplink-downlink configurations 1 and 2

| Allocation | Relative power level [dB] |  |  | NPDCCH and NPDSCH Data |
| --- | --- | --- | --- | --- |
|  | Subframe |  |  |  |
|  | 0, 5, 9 | 1, 6Note 5 | 3, 4, 8Note 3 |  |
| 0 | 0 (Note 2) | 0 | 0 | Note 1 |
| NOTE 1: This physical resource block is assigned to a virtual UE for transmission of NPDCCH or NPDSCH; the data transmitted over the NOCNG NPDCCH or NPDSCH shall be uncorrelated QPSK modulated pseudo random data. The parameter is used to scale the power of NPDCCH and NPDSCH. If two transmit antennas with NBRS are used in the test, the NPDCCH and NPDSCH parts of NOCNG shall be transmitted to the virtual users by both transmit antennas with NBRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the NPDCCH and NPDSCH parts of NOCNG is equal between both the transmit antennas with NBRS used in the test. The antenna transmission modes are specified in clause 7.1 in 3GPP TS 36.213. NOTE 2: Value of is applicable to PRBs not used for transmission of NPSS, NSSS and NPBCH in anchor cell (= 0). NOTE 3: Subframes available for DL transmission depends on the Uplink-Downlink configuration in Table 4.2-2 in 3GPP TS 36.211 [16]. NOTE 4: SI transmissions, and NPDCCH and NPDSCH for the UE under test, have precedence over NOCNG. NOTE 5: Whether NPDSCH transmissions are carried out in DwPTS depends on the special subframe configuration in use. |  |  |  |  |

# D.4	OCNG Patterns for V2X sidelink

## D.4.0

The following V2X sidelink OCNG patterns (VOCNG) are used for modelling allocations to virtual V2X UEs (which are not under test). The OCNG pattern for each subframe specifies the allocations that shall be filled with OCNG, and furthermore, the relative power level of each such allocation.

In each test case VOCNG is expressed by parameters VOCNG_RA and VOCNG_RB which together with a relative power level () specifies the PSSCH EPRE-to-RS EPRE ratios in OFDM symbols without and with reference symbols, respectively. The relative power, which is used for modelling boosting per virtual V2X UE allocation, is expressed by:

where  denotes the relative power level of the i:th virtual V2X UE. The parameter settings of VOCNG_RA, VOCNG_RB, and the set of relative power levels are chosen such that when also taking allocations to the UE under test into account, as given by a PSSCH reference channel.

Moreover the VOCNG pattern is accompanied by a PSCCH reference channel which specifies the control region. The number of PSCCH OFDM symbols in all subframes is the same as specified in the RMC used in the test.

## D.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz

Table D.4.1-1: VOP.1 HD: OCNG TDD Pattern 1

| Allocation | Relative power level [dB] | PSCCH and PSSCH Data |
| --- | --- | --- |
|  | Subframe Note 2 |  |
|  | 0 - 19 |  |
| 0 - 4 | 18 | Note 1 |
| 10-25 | 18 |  |
| NOTE 1: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PSSCH per virtual UE; the data transmitted over the OCNG PSSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameter is used to scale the power of PSSCH. NOTE 2: Value of is applicable to subframes not used for V2X sidelink transmissions. |  |  |

## D.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz

Table D.4.2-1: VOP.2 HD: OCNG TDD Pattern 2

| Allocation | Relative power level [dB] | PSCCH and PSSCH Data |
| --- | --- | --- |
|  | Subframe Note 2 |  |
|  | 0 - 19 |  |
| 0 - 4 | 18 | Note 1 |
| 10-14 | 18 |  |
| 15 -19 | 8.1 |  |
| 20-25 | 18 |  |
| NOTE 1: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PSSCH per virtual UE; the data transmitted over the OCNG PSSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameter is used to scale the power of PSSCH. NOTE 2: Value of is applicable to subframes not used for V2X sidelink transmissions. |  |  |

Annex E (normative):
Cell configuration mapping

The cells used in 3GPP TS 36.521-3 do not correspond to the cells defined in 3GPP TS 36.508 [7] clause 4.4.2. Tables E-1, E-2, E-3, E-5, E-6, E-7 and E-8 describes the mapping between cells described in 3GPP TS 36.521-3 and those defined in 3GPP TS 36.508 [7]. Table E-4 describe the mapping between Ncells described in 3GPP TS 36.521-3 and those defined in 3GPP TS 36.508 [7] For each test case the cells as defined in 3GPP TS 36.508 [7] clause 4.4.2 and clause 8.1.4.2 are listed in one row. The test case shall apply the RF parameters as defined in 3GPP TS 36.521-3 according to the column heading.

NOTE:	For example if the second cell in a test case is an inter-frequency cell then Cell3 from TS 36.508 [7] clause 4.4.2 is used with the radio parameters as defined for Cell2 in 3GPP TS 36.521-3.

Table E-1: Cell configuration mapping for RRM testing

| Test Case | Description | 36.521-3 Cell1 | 36.521-3 Cell2 | 36.521-3 Cell3 | 36.521-3 Cell4 | CA Type (Note 2) |
| --- | --- | --- | --- | --- | --- | --- |
| 4.2.1 | RRC IDLE / E-UTRAN Cell Reselection / FDD - FDD cell re-selection intra frequency case | Cell1 | Cell11 |  |  |  |
| 4.2.2 | RRC IDLE / E-UTRAN Cell Reselection / TDD - TDD cell re-selection intra frequency case | Cell1 | Cell11 |  |  |  |
| 4.2.3 | RRC IDLE / E-UTRAN Cell Reselection / FDD - FDD cell re-selection inter frequency case | Cell6 | Cell23 |  |  |  |
| 4.2.4 | RRC IDLE / E-UTRAN Cell Reselection / FDD - TDD cell re-selection inter frequency case | Cell1 | Cell31 | Dual mode in single PLMN |  |  |
| 4.2.5 | RRC IDLE / E-UTRAN Cell Reselection / TDD - FDD cell re-selection inter frequency case | Cell31 | Cell1 |  |  |  |
| 4.2.6 | RRC IDLE / E-UTRAN Cell Reselection / TDD - TDD cell re-selection inter frequency case | Cell6 | Cell23 |  |  |  |
| 4.2.7 | RRC IDLE / E-UTRAN Cell Reselection / FDD - FDD Inter frequency case in the existence of non-allowed CSG cell | Cell23 | Cell6 | Cell3 |  |  |
| 4.2.8 | RRC IDLE / E-UTRAN Cell Reselection / TDD - TDD Inter frequency case in the existence of non-allowed CSG cell | Cell23 | Cell6 | Cell3 |  |  |
| 4.2.9 | RRC IDLE / E-UTRAN Cell Reselection / E-UTRAN FDD-FDD intra-frequency Cell Re-selection case for 5MHz Bandwidth | Cell1 | Cell11 |  |  |  |
| 4.2.10 | E-UTRAN FDD - FDD reselection using an increased number of carriers | Cell6 | (Note 9) | (Note 10) (Note15) | (Note 10) (Note15) |  |
| 4.2.11 | E-UTRAN TDD - TDD reselection using an increased number of carriers | Cell6 | (Note 9) | (Note 10) (Note15) | (Note 10) (Note15) |  |
| 4.2.12 | E-UTRAN FDD - FDD Intra frequency case for Cat-M1 UE in normal coverage | Cell1 | Cell11 |  |  |  |
| 4.2.13 | E-UTRAN HD - FDD Intra frequency case for Cat-M1 UE in normal coverage | Cell1 | Cell11 |  |  |  |
| 4.2.14 | E-UTRAN TDD - TDD Intra frequency case for Cat-M1 UE in normal coverage | Cell1 | Cell11 |  |  |  |
| 4.2.15 | E-UTRAN FDD - FDD Intra frequency case for Cat-M1 UE in enhanced coverage | Cell1 | Cell11 |  |  |  |
| 4.2.16 | E-UTRAN HD - FDD Intra frequency case for Cat-M1 UE in enhanced coverage | Cell1 | Cell11 |  |  |  |
| 4.2.17 | E-UTRAN TDD - TDD Intra frequency case for Cat-M1 UE in enhanced coverage | Cell1 | Cell11 |  |  |  |
| 4.2.20 | E-UTRAN FDD - FDD Intra frequency case for UE Category 1bis | Cell1 | Cell11 |  |  |  |
| 4.2.21 | E-UTRAN TDD - TDD Intra frequency case for UE Category 1bis | Cell1 | Cell11 |  |  |  |
| 4.2.22 | E-UTRAN FDD - FDD cell re-selection intra frequency case for UE configured with highSpeedEnhancedMeasFlag | Cell1 | Cell11 |  |  |  |
| 4.2.23 | E-UTRAN TDD - TDD cell re-selection intra frequency case for UE configured with highSpeedEnhancedMeasFlag | Cell1 | Cell11 |  |  |  |
| 4.3.1.1 | RRC IDLE / E-UTRAN to UTRAN Cell re-selection / E-UTRAN FDD - UTRAN FDD cell re-selection: UTRA is of higher priority | Cell3 | Cell9 |  |  |  |
| 4.3.1.2 | RRC IDLE / E-UTRAN to UTRAN Cell re-selection / E-UTRAN FDD - UTRAN FDD cell re-selection: UTRA is of lower priority | Cell3 | Cell9 |  |  |  |
| 4.3.1.3 | RRC IDLE / E-UTRAN to UTRAN Cell re-selection / E-UTRAN FDD - UTRAN FDD cell re-selection in fading propagation conditions: UTRA FDD is of lower priority | Cell3 | Cell9 |  |  |  |
| 4.3.1.4 | RRC IDLE / E-UTRAN to UTRAN Cell re-selection / E-UTRAN FDD - UTRAN FDD cell re-selection: UTRA FDD is of lower priority for 5MHz Bandwidth | Cell3 | Cell9 |  |  |  |
| 4.3.1.5 | Idle mode FDD to UTRA FDD interRAT reselection | Cell6 | (Note11) | (Note14) (Note15) |  |  |
| 4.3.2 | RRC IDLE / E-UTRAN to UTRAN Cell re-selection / E-UTRAN FDD - UTRAN TDD cell re-selection | Cell6 | Cell8 |  |  |  |
| 4.3.2A | E-UTRA FDD to UTRA TDD cell re-selection for IncMon | Cell6 | (Note11) | (Note 16) (Note15) |  |  |
| 4.3.3 | RRC IDLE / E-UTRAN to UTRAN Cell re-selection / E-UTRAN TDD - UTRAN FDD cell re-selection | Cell6 | Cell8 |  |  |  |
| 4.3.3A | Idle mode TDD to UTRA FDD interRAT reselection | Cell6 | (Note11) | (Note 14) (Note 15) |  |  |
| 4.3.4.1 | RRC IDLE / E-UTRAN to UTRAN Cell re-selection / E-UTRAN TDD - UTRAN TDD cell re-selection: UTRA is of higher priority | Cell6 | Cell8 |  |  |  |
| 4.3.4.2 | RRC IDLE / E-UTRAN to UTRAN Cell re-selection / E-UTRAN TDD - UTRAN TDD cell re-selection: UTRA is of lower priority | Cell6 | Cell8 |  |  |  |
| 4.3.4.4 | E-UTRA TDD to UTRA TDD cell re-selection for IncMon | Cell6 | (Note11) | (Note 16) (Note 15) |  |  |
| 4.3.4.3 | RRC IDLE / E-UTRAN to UTRAN Cell re-selection /EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority | Cell3 | Cell9 |  |  |  |
| 4.4.1 | RRC IDLE / E-UTRAN to GSM Cell re-selection / E-UTRAN FDD - GSM cell re-selection | Cell1 | Cell26 |  |  |  |
| 4.4.2 | RRC IDLE / E-UTRAN to GSM Cell re-selection / E-UTRAN TDD - GSM cell re-selection | Cell1 | Cell26 |  |  |  |
| 4.5.1.1 | RRC IDLE / E-UTRAN to HRPD Cell re-selection / E-UTRAN FDD - HRPD cell re-selection: HRPD is of lower priority | Cell1 | Cell15 |  |  |  |
| 4.5.2.1 | RRC IDLE / E-UTRAN to HRPD Cell re-selection / E-UTRAN TDD - HRPD Cell Reselection: HRPD is of Lower Priority | Cell1 | Cell15 |  |  |  |
| 4.6.1.1 | RRC IDLE / E-UTRAN to cdma2000 1xRTT Cell re-selection / E-UTRAN FDD - cdma2000 1xRTT cell re-selection: cdma2000 1x is of lower priority | Cell1 | Cell19 |  |  |  |
| 4.6.2.1 | RRC IDLE / E-UTRAN to cdma2000 1xRTT Cell re-selection / E-UTRAN TDD - cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority | Cell1 | Cell19 |  |  |  |
| 5.1.1 | RRC CONNECTED / E-UTRAN Handover / FDD - FDD / Intra frequency case | Cell1 | Cell2 |  |  |  |
| 5.1.2 | RRC CONNECTED / E-UTRAN Handover / TDD - TDD / Intra frequency case | Cell1 | Cell2 |  |  |  |
| 5.1.3 | RRC CONNECTED / E-UTRAN Handover / FDD - FDD / Inter frequency case | Cell6 | Cell3 |  |  |  |
| 5.1.3_2 | E-UTRAN FDD - FDD Inter frequency handover for UE Category 1bis | Cell6 | Cell3 |  |  |  |
| 5.1.4 | RRC CONNECTED / E-UTRAN Handover / TDD - TDD / Inter frequency case | Cell6 | Cell3 |  |  |  |
| 5.1.4_2 | E-UTRAN TDD - TDD Inter frequency handover for UE Category 1bis | Cell6 | Cell3 |  |  |  |
| 5.1.5 | RRC CONNECTED / E-UTRAN Handover / FDD - FDD / Inter frequency case: unknown target cell | Cell6 | Cell3 |  |  |  |
| 5.1.5_2 | E-UTRAN FDD - FDD Inter frequency handover: unknown target cell for UE Category 1bis | Cell6 | Cell3 |  |  |  |
| 5.1.6 | RRC CONNECTED / E-UTRAN Handover / TDD - TDD / Inter frequency case: unknown target cell | Cell6 | Cell3 |  |  |  |
| 5.1.6_2 | E-UTRAN TDD - TDD Inter frequency handover; unknown Target Cell for UE Category 1bis | Cell6 | Cell3 |  |  |  |
| 5.1.7 | RRC CONNECTED / E-UTRAN Handover / FDD - TDD / Inter frequency case | Cell1 | Cell10 | Dual mode in multiple PLMN |  |  |
| 5.1.7_2 | E-UTRAN FDD - TDD Inter frequency handover for UE Category 1bis | Cell1 | Cell10 | Dual mode in multiple PLMN |  |  |
| 5.1.8 | RRC CONNECTED / E-UTRAN Handover / TDD - FDD / Inter frequency case | Cell10 | Cell1 |  |  |  |
| 5.1.8_2 | E-UTRAN TDD - FDD Inter frequency handover for UE Category 1bis | Cell10 | Cell1 |  |  |  |
| 5.1.9 | RRC CONNECTED / E-UTRAN Handover / FDD - FDD / Intra frequency handover for 5MHz Bandwidth | Cell1 | Cell2 |  |  |  |
| 5.1.10 | E-UTRAN FDD-FDD Handover intra frequency handover for UE category 0 | Cell1 | Cell2 |  |  |  |
| 5.1.11 | E-UTRAN HD-FDD Handover intra frequency handover for UE category 0 | Cell1 | Cell2 |  |  |  |
| 5.1.12 | E-UTRAN TDD-TDD Handover intra frequency handover for UE category 0 | Cell1 | Cell2 |  |  |  |
| 5.1.13 | E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA | Cell1 | Cell2 |  |  |  |
| 5.1.14 | E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA | Cell1 | Cell2 |  |  |  |
| 5.1.15 | E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA | Cell1 | Cell2 |  |  |  |
| 5.1.19 | E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis | Cell1 | Cell2 |  |  |  |
| 5.1.20 | E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis | Cell1 | Cell2 |  |  |  |
| 5.1.28 | E-UTRAN HD-FDD inter frequency handover for CE UEs in CEModeA | Cell6 | Cell3 |  |  |  |
| 5.1.34 | E-UTRAN HD-FDD intra frequency handover for CE UEs in CEModeA without SFN acquisition | Cell6 | Cell3 |  |  |  |
| 5.1.41 | E-UTRAN FDD – FDD Intra-band Inter-frequency sync DAPS handover | Cell6 | Cell3 |  |  |  |
| 5.1.42 | E-UTRAN FDD – FDD Intra-band Inter-frequency async DAPS handover | Cell6 | Cell3 |  |  |  |
| 5.1.43 | E-UTRAN FDD – FDD Inter-band Inter-frequency sync DAPS handover | Cell1 | Cell10 |  |  |  |
| 5.1.44 | E-UTRAN FDD – FDD Inter-band Inter-frequency async DAPS handover | Cell1 | Cell10 |  |  |  |
| 5.1.45 | E-UTRAN FDD - FDD Intra frequency DAPS handover | Cell1 | Cell2 |  |  |  |
| 5.1.46 | E-UTRAN TDD - TDD Intra frequency DAPS handover | Cell1 | Cell2 |  |  |  |
| 5.1.53 | E-UTRAN TDD – TDD Intra-band Inter-frequency sync DAPS handover | Cell6 | Cell3 |  |  |  |
| 5.1.54 | E-UTRAN TDD – TDD Inter-band Inter-frequency sync DAPS handover | Cell1 | Cell10 |  |  |  |
| 5.1.55 | E-UTRAN FDD - TDD inter-band inter-frequency synchronous DAPS handover | Cell1 | Cell10 |  |  |  |
| 5.1.56 | E-UTRAN TDD - FDD inter-band inter-frequency synchronous DAPS handover | Cell1 | Cell10 |  |  |  |
| 5.1.57 | E-UTRAN FDD – TDD Inter-band Inter-frequency async DAPS handover | Cell1 | Cell10 |  |  |  |
| 5.1.58 | E-UTRAN TDD – FDD Inter-band Inter-frequency async DAPS handover | Cell1 | Cell10 |  |  |  |
| 5.1.47 | E-UTRAN FDD - FDD Intra frequency conditional handover | Cell1 | Cell2 |  |  |  |
| 5.1.48 | E-UTRAN TDD - TDD Intra frequency conditional handover | Cell1 | Cell2 |  |  |  |
| 5.1.49 | E-UTRAN FDD - FDD Inter frequency conditional handover | Cell6 | Cell3 |  |  |  |
| 5.1.50 | E-UTRAN TDD - TDD Inter frequency conditional handover | Cell6 | Cell3 |  |  |  |
| 5.1.51 | E-UTRAN FDD - TDD Inter frequency conditional handover | Cell1 | Cell10 |  |  |  |
| 5.1.52 | E-UTRAN TDD - FDD Inter frequency conditional handover | Cell10 | Cell1 |  |  |  |
| 5.2.1 | RRC CONNECTED / Handover from E-UTRAN to other RATs / From E-UTRAN to UTRAN / E-UTRAN FDD - UTRAN FDD handover | Cell3 | Cell9 |  |  |  |
| 5.2.2 | RRC CONNECTED / Handover from E-UTRAN to other RATs / From E-UTRAN to UTRAN / E-UTRAN TDD - UTRAN FDD handover | Cell6 | Cell8 |  |  |  |
| 5.2.3 | RRC CONNECTED / Handover from E-UTRAN to other RATs / From E-UTRAN to GSM / E-UTRAN FDD - GSM handover | Cell1 | Cell26 |  |  |  |
| 5.2.4 | RRC CONNECTED / Handover from E-UTRAN to other RATs / From E-UTRAN to UTRAN / E-UTRAN TDD - UTRAN TDD handover | Cell3 | Cell9 |  |  |  |
| 5.2.5 | RRC CONNECTED / Handover from E-UTRAN to other RATs / From E-UTRAN to UTRAN / E-UTRAN FDD - UTRAN TDD handover | Cell6 | Cell8 |  |  |  |
| 5.2.6 | RRC CONNECTED / Handover from E-UTRAN to other RATs / From E-UTRAN to GSM / E-UTRA TDD - GSM handover | Cell1 | Cell26 |  |  |  |
| 5.2.7 | RRC CONNECTED / Handover from E-UTRAN to other RATs / E-UTRAN FDD - UTRAN FDD handover: unknown target cell | Cell3 | Cell9 |  |  |  |
| 5.2.8 | RRC CONNECTED / Handover from E-UTRAN to other RATs / E-UTRAN FDD - GSM handover: unknown target cell | Cell1 | Cell26 |  |  |  |
| 5.2.9 | RRC CONNECTED / Handover from E-UTRAN to other RATs / E-UTRAN TDD - GSM Handover: unknown target cell | Cell1 | Cell26 |  |  |  |
| 5.2.10 | RRC CONNECTED / Handover from E-UTRAN to other RATs / E-UTRAN TDD - UTRAN TDD HO test: unknown target cell | Cell6 | Cell8 |  |  |  |
| 5.2.11 | RRC CONNECTED / Handover from E-UTRAN to other RATs / E-UTRAN FDD - UTRAN FDD handover for 5MHz Bandwidth | Cell3 | Cell9 |  |  |  |
| 5.3.1 | RRC CONNECTED / Handover from E-UTRAN to non-3GPP RATs / E-UTRAN FDD - HRPD handover | Cell1 | Cell15 |  |  |  |
| 5.3.2 | RRC CONNECTED / Handover from E-UTRAN to non-3GPP RATs / E-UTRAN FDD - cdma2000 1xRTT handover | Cell1 | Cell19 |  |  |  |
| 5.3.3 | RRC CONNECTED / Handover from E-UTRAN to non-3GPP RATs / E-UTRAN FDD - HRPD Handover: unknown target cell | Cell1 | Cell15 |  |  |  |
| 5.3.4 | RRC CONNECTED / Handover from E-UTRAN to non-3GPP RATs / E-UTRAN FDD - cdma2000 1xRTT Handover: unknown target cell | Cell1 | Cell19 |  |  |  |
| 5.3.5 | RRC CONNECTED / Handover from E-UTRAN to non-3GPP RATs / E-UTRAN TDD-HRPD Handover | Cell1 | Cell15 |  |  |  |
| 5.3.6 | RRC CONNECTED / Handover from E-UTRAN to non-3GPP RATs / E-UTRAN TDD-cdma2000 1X Handover | Cell1 | Cell19 |  |  |  |
| 6.1.1 | RRC Connection Mobility Control / E-UTRAN FDD Intra-frequency RRC Re-establishment | Cell1 | Cell2 |  |  |  |
| 6.1.1_2 | RRC Connection Mobility Control / E-UTRAN FDD Intra-frequency RRC Re-establishment for UE Category 1bis | Cell1 | Cell2 |  |  |  |
| 6.1.2 | RRC Connection Mobility Control / E-UTRAN FDD Inter-frequency RRC Re-establishment | Cell6 | Cell3 |  |  |  |
| 6.1.2_2 | RRC Connection Mobility Control / E-UTRAN FDD Inter-frequency RRC Re-establishment for UE Category 1bis | Cell6 | Cell3 |  |  |  |
| 6.1.3 | RRC Connection Mobility Control / E-UTRAN TDD Intra-frequency RRC Re-establishment | Cell1 | Cell2 |  |  |  |
| 6.1.3_2 | RRC Connection Mobility Control / E-UTRAN TDD Intra-frequency RRC Re-establishment for UE Category 1bis | Cell1 | Cell2 |  |  |  |
| 6.1.4 | RRC Connection Mobility Control / E-UTRAN TDD Inter-frequency RRC Re-establishment | Cell6 | Cell3 |  |  |  |
| 6.1.4_2 | RRC Connection Mobility Control / E-UTRAN TDD Inter-frequency RRC Re-establishment for UE Category 1bis | Cell6 | Cell3 |  |  |  |
| 6.1.5 | RRC Connection Mobility Control / E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz Bandwidth | Cell1 | Cell2 |  |  |  |
| 6.1.8 | E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0 | Cell1 | Cell2 |  |  |  |
| 6.1.9 | E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | Cell1 | Cell2 |  |  |  |
| 6.1.10 | E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | Cell1 | Cell2 |  |  |  |
| 6.1.11 | E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | Cell1 | Cell2 |  |  |  |
| 6.1.12 | E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 6.1.13 | E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 6.1.14 | E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 6.1.18 | E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for CE UE in CEModeA | Cell6 | Cell3 |  |  |  |
| 6.2.1 | RRC Connection Mobility Control / Random Access / E-UTRAN FDD - Contention Based Random Access | Cell1 |  |  |  |  |
| 6.2.1_2 | RRC Connection Mobility Control / Random Access / E-UTRAN FDD - Contention Based Random Access for UE Category 1bis | Cell1 |  |  |  |  |
| 6.2.2 | RRC Connection Mobility Control / Random Access / E-UTRAN FDD - Non-Contention Based Random Access | Cell1 |  |  |  |  |
| 6.2.2_2 | RRC Connection Mobility Control / Random Access / E-UTRAN FDD - Non-Contention Based Random Access for UE Category 1bis | Cell1 |  |  |  |  |
| 6.2.3 | RRC Connection Mobility Control / Random Access / E-UTRAN TDD - Contention Based Random Access | Cell1 |  |  |  |  |
| 6.2.3_2 | RRC Connection Mobility Control / Random Access / E-UTRAN TDD - Contention Based Random Access for UE Category 1bis | Cell1 |  |  |  |  |
| 6.2.4 | RRC Connection Mobility Control / Random Access / E-UTRAN TDD - Non-Contention Based Random Access | Cell1 |  |  |  |  |
| 6.2.4_2 | RRC Connection Mobility Control / Random Access / E-UTRAN TDD - Non-Contention Based Random Access for UE Category 1bis | Cell1 |  |  |  |  |
| 6.2.5 | RRC Connection Mobility Control / Random Access / E-UTRAN FDD - Contention Based Random Access Test for 5MHz Bandwidth | Cell1 |  |  |  |  |
| 6.2.6 | RRC Connection Mobility Control / Random Access / E-UTRAN FDD - Non-Contention Based Random Access Test for 5MHz Bandwidth | Cell1 |  |  |  |  |
| 6.2.7 | E-UTRAN FDD - Non-Contention Based Random Access Test For SCell in sTAG | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 6.2.8 | E-UTRAN TDD - Non-Contention Based Random Access Test For SCell in sTAG | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 6.2.10 | E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage | Cell1 |  |  |  |  |
| 6.2.11 | E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage | Cell1 |  |  |  |  |
| 6.2.12 | RRC Connection Mobility Control / Random Access / E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage | Cell1 |  |  |  |  |
| 6.2.13 | E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage | Cell1 |  |  |  |  |
| 6.2.14 | E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage | Cell1 |  |  |  |  |
| 6.2.15 | E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage | Cell1 |  |  |  |  |
| 6.3.1 | RRC Connection Mobility Control / RRC Connection Release with Redirection / Redirection from E-UTRAN FDD to UTRAN FDD | Cell3 | Cell9 |  |  |  |
| 6.3.1_2 | RRC Connection Mobility Control / RRC Connection Release with Redirection / Redirection from E-UTRAN FDD to UTRAN FDD for UE Category 1bis | Cell3 | Cell9 |  |  |  |
| 6.3.2 | RRC Connection Mobility Control / RRC Connection Release with Redirection / Redirection from E-UTRAN TDD to UTRAN FDD | Cell6 | Cell8 |  |  |  |
| 6.3.2_2 | RRC Connection Mobility Control / RRC Connection Release with Redirection / Redirection from E-UTRAN TDD to UTRAN FDD for UE Category 1bis | Cell6 | Cell8 |  |  |  |
| 6.3.3 | RRC Connection Mobility Control / RRC Connection Release with Redirection / Redirection from E-UTRAN FDD to GERAN when System Information is provided | Cell1 | Cell26 |  |  |  |
| 6.3.3_2 | RRC Connection Mobility Control / RRC Connection Release with Redirection / Redirection from E-UTRAN FDD to GERAN when System Information is provided for UE Category 1bis | Cell1 | Cell26 |  |  |  |
| 6.3.4 | RRC Connection Mobility Control / RRC Connection Release with Redirection / Redirection from E-UTRAN TDD to GERAN when System Information is provided | Cell1 | Cell26 |  |  |  |
| 6.3.4_2 | RRC Connection Mobility Control / RRC Connection Release with Redirection / Redirection from E-UTRAN TDD to GERAN when System Information is provided for UE Category 1bis | Cell1 | Cell26 |  |  |  |
| 6.3.5 | RRC Connection Mobility Control / RRC Connection Release with Redirection / E-UTRA TDD RRC connection release redirection to UTRA TDD | Cell3 | Cell9 |  |  |  |
| 6.3.5_2 | RRC Connection Mobility Control / RRC Connection Release with Redirection / E-UTRA TDD RRC connection release redirection to UTRA TDD for UE Category 1bis | Cell3 | Cell9 |  |  |  |
| 6.3.6 | RRC Connection Mobility Control / RRC Connection Release with Redirection / E-UTRA FDD RRC connection release redirection to UTRA TDD | Cell6 | Cell8 |  |  |  |
| 6.3.6_2 | RRC Connection Mobility Control / RRC Connection Release with Redirection / E-UTRA FDD RRC connection release redirection to UTRA TDD for UE Category 1bis | Cell6 | Cell8 |  |  |  |
| 6.3.7 | RRC Connection Mobility Control / RRC Connection Release with Redirection / E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided | Cell3 | Cell9 |  |  |  |
| 6.3.7_2 | RRC Connection Mobility Control / RRC Connection Release with Redirection / E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided for UE Category 1bis | Cell3 | Cell9 |  |  |  |
| 6.3.8 | RRC Connection Mobility Control / RRC Connection Release with Redirection / E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided | Cell6 | Cell8 |  |  |  |
| 6.3.8_2 | RRC Connection Mobility Control / RRC Connection Release with Redirection / E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided for UE Category 1bis | Cell6 | Cell8 |  |  |  |
| 6.3.9 | RRC Connection Mobility Control / RRC Connection Release with Redirection / Redirection from E-UTRAN FDD to UTRAN FDD without System Information | Cell3 | Cell9 |  |  |  |
| 6.3.9_2 | RRC Connection Mobility Control / RRC Connection Release with Redirection / Redirection from E-UTRAN FDD to UTRAN FDD without System Information for UE Category 1bis | Cell3 | Cell9 |  |  |  |
| 6.3.10 | RRC Connection Mobility Control / RRC Connection Release with Redirection / Redirection from E-UTRAN FDD to GERAN when System Information is not provided | Cell1 | Cell26 |  |  |  |
| 6.3.10_2 | RRC Connection Mobility Control / RRC Connection Release with Redirection / Redirection from E-UTRAN FDD to GERAN when System Information is not provided for UE Category 1bis | Cell1 | Cell26 |  |  |  |
| 6.3.11 | RRC Connection Mobility Control / RRC Connection Release with Redirection / Redirection from E-UTRAN TDD to GERAN when System Information is not provided | Cell1 | Cell26 |  |  |  |
| 6.3.11_2 | RRC Connection Mobility Control / RRC Connection Release with Redirection / Redirection from E-UTRAN TDD to GERAN when System Information is not provided for UE Category 1bis | Cell1 | Cell26 |  |  |  |
| 6.3.12 | RRC Connection Mobility Control / RRC Connection Release with Redirection / E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided | Cell6 | Cell8 |  |  |  |
| 6.3.12_2 | RRC Connection Mobility Control / RRC Connection Release with Redirection / E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided for UE Category 1bis | Cell6 | Cell8 |  |  |  |
| 7.1.1 | E-UTRAN FDD-UE Transmit Timing Accuracy | Cell1 |  |  |  |  |
| 7.1.1_2 | E-UTRAN FDD - UE Transmit Timing Accuracy for UE category 1bis | Cell1 |  |  |  |  |
| 7.1.2 | E-UTRAN TDD-UE Transmit Timing Accuracy | Cell1 |  |  |  |  |
| 7.1.2_2 | E-UTRAN TDD - UE Transmit Timing Accuracy for UE category 1bis | Cell1 |  |  |  |  |
| 7.1.3 | E-UTRAN FDD - UE Transmit Timing Accuracy Tests for SCell | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 7.1.3_1 | E-UTRAN FDD - UE Transmit Timing Accuracy Tests for SCell (Release 12 and forward) | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 7.1.4 | E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 7.1.4A | E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz +10 MHz bandwidth | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 7.1.4_1 | E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell (Release 12 and forward) | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 7.1.5 | E-UTRAN FDD - UE Transmit Timing Accuracy Tests for 5MHz Bandwidth | Cell1 |  |  |  |  |
| 7.1.6 | E-UTRAN FDD - UE Transmit Timing Accuracy Tests for SCell in sTAG | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 7.1.7 | E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 7.1.7A | E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz bandwidth | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 7.1.7B | E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz bandwidth | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 7.1.8 | Void |  |  |  |  |  |
| 7.1.9 | Void |  |  |  |  |  |
| 7.1.10 | E-UTRAN FDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA | Cell1 |  |  |  |  |
| 7.1.11 | E-UTRAN HD-FDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA | Cell1 |  |  |  |  |
| 7.1.12 | E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA | Cell1 |  |  |  |  |
| 7.1.14 | E-UTRAN FDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB | Cell1 |  |  |  |  |
| 7.1.15 | E-UTRAN HD-FDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB | Cell1 |  |  |  |  |
| 7.1.16 | E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB | Cell1 |  |  |  |  |
| 7.1.21 | E-UTRAN FDD - UE Transmit Timing Accuracy Tests for CE UE in CEModeA | Cell1 |  |  |  |  |
| 7.1.22 | E-UTRAN HD-FDD - UE Transmit Timing Accuracy Tests for CE UE in CEModeA | Cell1 |  |  |  |  |
| 7.1.23 | E-UTRAN TDD - UE Transmit Timing Accuracy Tests for CE UE in CEModeA | Cell1 |  |  |  |  |
| 7.1.24 | E-UTRAN FDD - UE Transmit Timing Accuracy Tests for CE UE in CEModeB | Cell1 |  |  |  |  |
| 7.2.1 | E-UTRAN FDD-UE Timing Advance Adjustment Accuracy | Cell1 |  |  |  |  |
| 7.2.1_2 | E-UTRAN FDD - UE Timing Advance Adjustment Accuracy for UE category 1bis | Cell1 |  |  |  |  |
| 7.2.2 | E-UTRAN TDD-UE Timing Advance Adjustment Accuracy | Cell1 |  |  |  |  |
| 7.2.2_2 | E-UTRAN TDD - UE Timing Advance Adjustment Accuracy for UE category 1bis | Cell1 |  |  |  |  |
| 7.2.3 | E-UTRAN FDD - UE Timing Advance Adjustment Accuracy Test for 5MHz Bandwidth | Cell1 |  |  |  |  |
| 7.2.4 | E-UTRAN FDD - UE Timing Advance Adjustment Accuracy Test For SCell in sTAG | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 7.2.5 | E-UTRAN TDD - UE Timing Advance Adjustment Accuracy Test For SCell in sTAG | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 7.2.5A | E-UTRAN TDD - UE Timing Advance Adjustment Accuracy Test for SCell in sTAG for 20MHz +20MHz bandwidth | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 7.2.5B | E-UTRAN TDD - UE Timing Advance Adjustment Accuracy Test for SCell in sTAG for 20MHz +10MHz bandwidth | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 7.2.6 | E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA | Cell1 |  |  |  |  |
| 7.2.7 | E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA | Cell1 |  |  |  |  |
| 7.2.8 | E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA | Cell1 |  |  |  |  |
| 7.2.10 | E-UTRAN FDD Timing Advance Adjustment Accuracy Test in CEModeB | Cell1 |  |  |  |  |
| 7.2.11 | E-UTRAN HD-FDD Timing Advance Adjustment Accuracy Test in CEModeB | Cell1 |  |  |  |  |
| 7.2.12 | E-UTRAN TDD Timing Advance Adjustment Accuracy Test in CEModeB | Cell1 |  |  |  |  |
| 7.3.1 | E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync | Cell1 |  |  |  |  |
| 7.3.1_1 | E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync with 4 Rx antenna ports | Cell1 |  |  |  |  |
| 7.3.2 | E-UTRAN FDD Radio Link Monitoring Test for In-sync | Cell1 |  |  |  |  |
| 7.3.2_1 | E-UTRAN FDD Radio Link Monitoring Test for In-sync with 4 Rx antenna ports | Cell1 |  |  |  |  |
| 7.3.3 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync | Cell1 |  |  |  |  |
| 7.3.3_1 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync with 4 Rx antenna ports | Cell 1 |  |  |  |  |
| 7.3.4 | E-UTRAN TDD Radio Link Monitoring Test for In-sync | Cell1 |  |  |  |  |
| 7.3.4_1 | E-UTRAN TDD Radio Link Monitoring Test for In-sync with 4 Rx antenna ports | Cell1 |  |  |  |  |
| 7.3.5 | E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX | Cell1 |  |  |  |  |
| 7.3.6 | E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX | Cell1 |  |  |  |  |
| 7.3.7 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX | Cell1 |  |  |  |  |
| 7.3.8 | E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX | Cell1 |  |  |  |  |
| 7.3.9 | E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non MBSFN ABS (eICIC) | Cell1 | Cell2 |  |  |  |
| 7.3.10 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non MBSFN ABS (eICIC) | Cell1 | Cell2 |  |  |  |
| 7.3.11 | E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with Non MBSFN ABS (eICIC) | Cell1 | Cell2 |  |  |  |
| 7.3.12 | E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with Non MBSFN ABS (eICIC) | Cell1 | Cell2 |  |  |  |
| 7.3.13 | E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Note 3) | Cell1 | Cell3 |  |  |  |
| 7.3.14 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Note 3) | Cell1 | Cell3 |  |  |  |
| 7.3.15 | E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Note 3) | Cell1 | Cell3 |  |  |  |
| 7.3.16 | E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Note 3) | Cell1 | Cell3 |  |  |  |
| 7.3.17 | E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS assistance information and Non MBSFN ABS (feICIC) (Note 3) | Cell1 | Cell3 | Cell4 |  |  |
| 7.3.18 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS assistance information and Non MBSFN ABS (feICIC) (Note 3) | Cell1 | Cell3 | Cell4 |  |  |
| 7.3.19 | E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and Non MBSFN ABS (feICIC) (Note 3) | Cell1 | Cell3 | Cell4 |  |  |
| 7.3.20 | E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and Non MBSFN ABS (feICIC) (Note 3) | Cell1 | Cell3 | Cell4 |  |  |
| 7.3.21 | E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and MBSFN ABS (feICIC) (Note 3) | Cell1 | Cell3 | Cell4 |  |  |
| 7.3.22 | E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and MBSFN ABS (feICIC) (Note 3) | Cell1 | Cell3 | Cell4 |  |  |
| 7.3.23 | E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth | Cell1 |  |  |  |  |
| 7.3.24 | E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth | Cell1 |  |  |  |  |
| 7.3.25 | E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth | Cell1 |  |  |  |  |
| 7.3.26 | E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE category 0 | Cell1 |  |  |  |  |
| 7.3.26_2 | E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 1bis | Cell1 |  |  |  |  |
| 7.3.27 | E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE category 0 | Cell1 |  |  |  |  |
| 7.3.27_2 | E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 1bis | Cell1 |  |  |  |  |
| 7.3.28 | E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0 | Cell1 |  |  |  |  |
| 7.3.28_2 | E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE Category 1bis | Cell1 |  |  |  |  |
| 7.3.29 | E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE category 0 | Cell1 |  |  |  |  |
| 7.3.29_2 | E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 1bis | Cell1 |  |  |  |  |
| 7.3.30 | E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE category 0 | Cell1 |  |  |  |  |
| 7.3.31 | E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE category 0 | Cell1 |  |  |  |  |
| 7.3.32 | E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0 | Cell1 |  |  |  |  |
| 7.3.33 | E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE category 0 | Cell1 |  |  |  |  |
| 7.3.34 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE category 0 | Cell1 |  |  |  |  |
| 7.3.34_2 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 1bis | Cell1 |  |  |  |  |
| 7.3.35 | E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0 | Cell1 |  |  |  |  |
| 7.3.35_2 | E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE Category 1bis | Cell1 |  |  |  |  |
| 7.3.36 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0 | Cell1 |  |  |  |  |
| 7.3.36_2 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE Category 1bis | Cell1 |  |  |  |  |
| 7.3.37 | E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0 | Cell1 |  |  |  |  |
| 7.3.37_2 | E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category 1bis | Cell1 |  |  |  |  |
| 7.3.38 | E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC | Cell1 | Cell10 |  |  |  |
| 7.3.39 | E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC | Cell1 | Cell10 |  |  |  |
| 7.3.40 | E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC | Cell1 | Cell10 |  |  |  |
| 7.3.41 | E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity | Cell1 | Cell10 |  |  |  |
| 7.3.42 | E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC | Cell1 | Cell10 |  |  |  |
| 7.3.43 | E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity | Cell1 | Cell10 |  |  |  |
| 7.3.44 | E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD | Cell1 | Cell10 |  |  |  |
| 7.3.45 | E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD | Cell1 | Cell10 |  |  |  |
| 7.3.46 | E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD | Cell1 | Cell10 |  |  |  |
| 7.3.47 | E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD | Cell1 | Cell10 |  |  |  |
| 7.3.48 | E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A | Cell1 |  |  |  |  |
| 7.3.49 | E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A | Cell1 |  |  |  |  |
| 7.3.50 | E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A | Cell1 |  |  |  |  |
| 7.3.51 | E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A | Cell1 |  |  |  |  |
| 7.3.52 | E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A | Cell1 |  |  |  |  |
| 7.3.53 | E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A | Cell1 |  |  |  |  |
| 7.3.54 | E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A | Cell1 |  |  |  |  |
| 7.3.55 | E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A | Cell1 |  |  |  |  |
| 7.3.56 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A | Cell1 |  |  |  |  |
| 7.3.57 | E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A | Cell1 |  |  |  |  |
| 7.3.58 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A | Cell1 |  |  |  |  |
| 7.3.59 | E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A | Cell1 |  |  |  |  |
| 7.3.69 | E-UTRAN HD-FDD Early Out-of-sync reporting Test for CE UE in CEMode A | Cell1 |  |  |  |  |
| 7.4.1 | E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC | Cell1 | Cell10 |  |  |  |
| 7.4.2 | E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC | Cell1 | Cell10 |  |  |  |
| 7.5.1 | E-UTRAN FDD - UE ProSe Direct Discovery Transmission Timing Accuracy Test | Cell 1 |  |  |  |  |
| 7.5.4 | E-UTRAN FDD - UE ProSe Direct Communication Transmission Timing Accuracy Test | Cell 1 |  |  |  |  |
| 7.6.1 | E-UTRAN FDD-TDD CA interruption at SRS carrier based switching | Cell 1 | Cell 10 |  |  |  |
| 7.6.2 | E-UTRAN TDD-TDD CA interruption at SRS carrier based switching | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 8.1.1 | UE Measurement Procedures / E-UTRAN FDD-FDD intra frequency event triggered reporting under fading propagation conditions in asynchronous cells | Cell1 | Cell2 |  |  |  |
| 8.1.2 | UE Measurement Procedures / E-UTRAN FDD-FDD intra frequency event triggered reporting under fading propagation conditions in synchronous cells | Cell1 | Cell2 |  |  |  |
| 8.1.3 | UE Measurement Procedures / E-UTRAN FDD-FDD intra frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX | Cell1 | Cell2 |  |  |  |
| 8.1.4 | Void |  |  |  |  |  |
| 8.1.5 | UE Measurement Procedures / E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | Cell1 | Cell2 |  |  |  |
| 8.1.6 | UE Measurement Procedures / E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX | Cell1 | Cell2 |  |  |  |
| 8.1.7 | UE Measurement Procedures / E-UTRAN FDD-FDD Intra-frequency event-triggered reporting under time domain measurement resource restriction with non-MBSFN ABS (eICIC) | Cell1 | Cell2 |  |  |  |
| 8.1.8 | E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) (Note 3) | Cell1 | Cell2 | Cell6 |  |  |
| 8.1.9 | E-UTRAN FDD-FDD intra frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth | Cell1 | Cell2 |  |  |  |
| 8.1.10 | E-UTRAN FDD-FDD intra frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for 5MHz bandwidth | Cell1 | Cell2 |  |  |  |
| 8.1.11 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0 | Cell1 | Cell2 |  |  |  |
| 8.1.11_2 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 1bis | Cell1 | Cell2 |  |  |  |
| 8.1.12 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0 | Cell1 | Cell2 |  |  |  |
| 8.1.12_2 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 1bis | Cell1 | Cell2 |  |  |  |
| 8.1.13 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0 | Cell1 | Cell2 |  |  |  |
| 8.1.13_2 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 1bis | Cell1 | Cell2 |  |  |  |
| 8.1.17 | E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells | Cell1 | Cell2 |  |  |  |
| 8.1.17_2 | E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 1bis | Cell1 | Cell2 |  |  |  |
| 8.1.18 | E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX | Cell1 | Cell2 |  |  |  |
| 8.1.18_2 | E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 1bis | Cell1 | Cell2 |  |  |  |
| 8.1.23 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA | Cell1 | Cell2 |  |  |  |
| 8.1.24 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA | Cell1 | Cell2 |  |  |  |
| 8.1.25 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | Cell1 | Cell2 |  |  |  |
| 8.1.26 | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA | Cell1 | Cell2 |  |  |  |
| 8.1.27 | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA | Cell1 | Cell2 |  |  |  |
| 8.1.28 | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | Cell1 | Cell2 |  |  |  |
| 8.1.29 | E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA | Cell1 | Cell2 |  |  |  |
| 8.1.30 | E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | Cell1 | Cell2 |  |  |  |
| 8.1.31 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 8.1.32 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 8.1.33 | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 8.1.34 | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 8.1.35 | E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 8.1.36 | E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 8.1.37 | -UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 8.1.38 | E-UTRAN HD- FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 8.1.39 | E-UTRAN HD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 8.1.40 | E-UTRAN FDD-FDD intra-frequency event triggered reporting for UE configured with highSpeedEnhancedMeasFlag in synchronous cells | Cell1 | Cell2 |  |  |  |
| 8.2.1 | UE Measurement Procedures / E-UTRAN TDD-TDD intra frequency event triggered reporting under fading propagation conditions in synchronous cells | Cell1 | Cell2 |  |  |  |
| 8.2.2 | UE Measurement Procedures / E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX | Cell1 | Cell2 |  |  |  |
| 8.2.3 | UE Measurement Procedures / E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | Cell1 | Cell2 |  |  |  |
| 8.2.4 | UE Measurement Procedures / E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX | Cell1 | Cell2 |  |  |  |
| 8.2.5 | UE Measurement Procedures / E-UTRAN TDD-TDD intra-frequency event-triggered Reporting under time domain measurement resource restriction with non-MBSFN ABS (eICIC) | Cell1 | Cell2 |  |  |  |
| 8.2.6 | E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) (Note 3) | Cell1 | Cell2 | Cell6 |  |  |
| 8.2.9 | E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 8.2.10 | E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 8.2.11 | E-UTRAN TDD-TDD intra-frequency event triggered reporting for UE configured with highSpeedEnhancedMeasFlag in synchronous cells | Cell1 | Cell11 |  |  |  |
| 8.3.1 | UE Measurement Procedures / E-UTRAN FDD-FDD inter frequency event triggered reporting under fading propagation conditions in asynchronous cells | Cell6 | Cell3 |  |  |  |
| 8.3.1_2 | UE Measurement Procedures / E-UTRAN FDD-FDD inter frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 1bis | Cell6 | Cell3 |  |  |  |
| 8.3.2 | UE Measurement Procedures / E-UTRAN FDD-FDD inter frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells | Cell6 | Cell3 |  |  |  |
| 8.3.2_2 | UE Measurement Procedures / E-UTRAN FDD-FDD inter frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells for UE category 1bis | Cell6 | Cell3 |  |  |  |
| 8.3.3 | UE Measurement Procedures / E-UTRAN FDD-FDD inter frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used | Cell6 | Cell3 |  |  |  |
| 8.3.3_2 | E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used for UE category 1bis | Cell6 | Cell3 |  |  |  |
| 8.3.4 | UE Measurement Procedures / E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | Cell6 | Cell3 |  |  |  |
| 8.3.5 | UE Measurement Procedures / E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX | Cell6 | Cell3 |  |  |  |
| 8.3.6 | UE Measurement Procedures / E-UTRAN FDD-FDD inter frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells | Cell6 | Cell3 |  |  |  |
| 8.3.7 | UE Measurement Procedures / E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group | Cell6 | (Note 8) | (Note 8) | (Note 8) |  |
| 8.3.8 | UE Measurement Procedures / FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, non DRX | Cell6 | (Note 9) | (Note 10) | (Note 10) |  |
| 8.3.9 | UE Measurement Procedures / FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX | Cell6 | (Note 9) | (Note 10) | (Note 10) |  |
| 8.3.12 | E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeA | Cell6 | Cell3 |  |  |  |
| 8.3.13 | E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeA | Cell6 | Cell3 |  |  |  |
| 8.3.14 | E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeB | Cell6 | Cell3 |  |  |  |
| 8.3.15 | E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeB | Cell6 | Cell3 |  |  |  |
| 8.3.16 | E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeA when DRX is used | Cell6 | Cell3 |  |  |  |
| 8.3.17 | E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeA in DRX | Cell6 | Cell3 |  |  |  |
| 8.3.18 | E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeB in DRX | Cell6 | Cell3 |  |  |  |
| 8.3.19 | E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeB in DRX | Cell6 | Cell3 |  |  |  |
| 8.4.1 | UE Measurement Procedures / E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in synchronous cells | Cell6 | Cell3 |  |  |  |
| 8.4.1_2 | UE Measurement Procedures / E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 1bis | Cell6 | Cell3 |  |  |  |
| 8.4.2 | UE Measurement Procedures / E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells | Cell6 | Cell3 |  |  |  |
| 8.4.2_2 | UE Measurement Procedures / E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells for UE category 1bis | Cell6 | Cell3 |  |  |  |
| 8.4.3 | UE Measurement Procedures / E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used | Cell6 | Cell3 |  |  |  |
| 8.4.3_2 | E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used for UE category 1bis | Cell6 | Cell3 |  |  |  |
| 8.4.4 | UE Measurement Procedures / E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | Cell6 | Cell3 |  |  |  |
| 8.4.5 | UE Measurement Procedures / E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX | Cell6 | Cell3 |  |  |  |
| 8.4.6 | UE Measurement Procedures / E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0 | Cell6 | Cell3 |  |  |  |
| 8.4.6_2 | UE Measurement Procedures / E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0 for UE category 1bis | Cell6 | Cell3 |  |  |  |
| 8.4.7 | UE Measurement Procedures / E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group | Cell6 | (Note 8) | (Note 8) | (Note 8) |  |
| 8.4.8 | UE Measurement Procedures / TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX | Cell6 | (Note 9) | (Note 10) | (Note 10) |  |
| 8.4.9 | TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX | Cell6 | (Note 9) | (Note 10) | (Note 10) |  |
| 8.4.12 | E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeA |  |  |  |  |  |
| 8.4.13 | E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeB |  |  |  |  |  |
| 8.4.14 | E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeA in DRX |  |  |  |  |  |
| 8.4.15 | E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeB in DRX |  |  |  |  |  |
| 8.5.1 | UE Measurement Procedures / E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions | Cell3 | Cell9 |  |  |  |
| 8.5.2 | UE Measurement Procedures / E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions | Cell3 | Cell9 |  |  |  |
| 8.5.3 | UE Measurement Procedures / E-UTRAN FDD - UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions | Cell3 | Cell9 |  |  |  |
| 8.5.4 | UE Measurement Procedures / E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions | Cell3 | Cell9 |  |  |  |
| 8.5.6 | UE Measurement Procedures / E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions | Cell3 | Cell9 |  |  |  |
| 8.5.7 | UE Measurement Procedures / E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions for 5MHz bandwidth | Cell3 | Cell9 |  |  |  |
| 8.5.8 | E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX | Cell6 | (Note 11) | (Note 12) | (Note 12) |  |
| 8.6.1 | UE Measurement Procedures / E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions | Cell1 | Cell8 |  |  |  |
| 8.6.3 | E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX | Cell6 | (Note 11) | (Note 12) | (Note 12) |  |
| 8.7.1 | UE Measurement Procedures / E-UTRAN TDD - UTRAN TDD event triggered reporting under fading propagation conditions | Cell6 | Cell8 |  |  |  |
| 8.7.2 | UE Measurement Procedures / E-UTRAN TDD - UTRAN TDD cell search when DRX is used under fading propagation conditions | Cell6 | Cell8 |  |  |  |
| 8.7.3 | UE Measurement Procedures / E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting under AWGN propagation conditions | Cell3 | Cell9 |  |  |  |
| 8.7.4 | UE Measurement Procedures / E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions | Cell3 | Cell9 |  |  |  |
| 8.7.5 | E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX | Cell6 | (Note 11) | (Note 12) | (Note 13) |  |
| 8.7A.1 | E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX | Cell6 | (Note 11) | (Note 12) | (Note 13) |  |
| 8.8.1 | UE Measurement Procedures / E-UTRAN FDD - GSM event triggered reporting in AWGN | Cell6 | Cell26 |  |  |  |
| 8.8.2 | UE Measurement Procedures / E-UTRAN FDD - GSM event triggered reporting when DRX is used in AWGN | Cell6 | Cell26 |  |  |  |
| 8.9.1 | UE Measurement Procedures / E-UTRAN FDD - UTRAN TDD event triggered reporting under fading propagation conditions | Cell6 | Cell8 |  |  |  |
| 8.9.2 | UE Measurement Procedures / E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions | Cell6 | Cell8 |  |  |  |
| 8.10.1 | UE Measurement Procedures / E-UTRAN TDD - GSM event triggered reporting in AWGN | Cell6 | Cell26 |  |  |  |
| 8.10.2 | UE Measurement Procedures / E-UTRAN TDD - GSM event triggered reporting when DRX is used in AWGN | Cell6 | Cell26 |  |  |  |
| 8.11.1 | UE Measurement Procedures / Monitoring of multiple layers / E-UTRAN FDD - E-UTRAN FDD and E-UTRAN FDD Inter-frequency event triggered reporting under fading propagation conditions | Cell1 | Cell3 | Cell6 |  |  |
| 8.11.2 | UE Measurement Procedures / Monitoring of multiple layers / E-UTRAN TDD - E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions | Cell1 | Cell3 | Cell6 |  |  |
| 8.11.3 | UE Measurement Procedures / Monitoring of multiple layers / InterRAT E-UTRA FDD to E-UTRA FDD and UTRA FDD cell search | Cell1 | Cell6 | Cell8 |  |  |
| 8.11.4 | UE Measurement Procedures / Monitoring of multiple layers / InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search | Cell1 | Cell6 | Cell8 |  |  |
| 8.11.5 | UE Measurement Procedures / Combined E-UTRAN FDD - E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions | Cell6 | Cell3 | Cell24 |  |  |
| 8.11.6 | UE Measurement Procedures / Combined E-UTRAN TDD - E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions | Cell6 | Cell3 | Cell24 |  |  |
| 8.14.1 | UE Measurement Procedures / E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells | Cell10 | Cell1 |  |  |  |
| 8.14.2 | UE Measurement Procedures / E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells | Cell10 | Cell1 |  |  |  |
| 8.14.3 | UE Measurement Procedures / E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | Cell10 | Cell1 |  |  |  |
| 8.15.1 | UE Measurement Procedures / E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells | Cell1 | Cell10 |  |  |  |
| 8.15.2 | UE Measurement Procedures / E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells | Cell1 | Cell10 |  |  |  |
| 8.15.3 | UE Measurement Procedures / E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | Cell1 | Cell10 |  |  |  |
| 8.16.1 | UE Measurement Procedures / E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.16.2 | UE Measurement Procedures / E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.16.3 | UE Measurement Procedures / E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.16.4 | UE Measurement Procedures / E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.16.5 | UE Measurement Procedures / E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.16.6 | UE Measurement Procedures / E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.16.7 | UE Measurement Procedures / E-UTRAN FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.16.8 | UE Measurement Procedures / E-UTRAN TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.16.9 | E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz | Cell6 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.16.10 | E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz | Cell6 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.16.11 | E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz | Cell6 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.16.12 | E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz | Cell6 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.16.13 | E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz+5MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.16.14 | E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5MHz+5MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.16.15 | E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5MHz+5MHz bandwidth | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.16.16 | E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5MHz+5MHz bandwidth | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.16.17 | E-UTRAN FDD activation and deactivation of known SCell in non-DRX | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 8.16.17A | E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz +20MHz bandwidth | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 8.16.18 | E-UTRAN TDD activation and deactivation of known SCell in non-DRX | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 8.16.18A | E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz +20MHz bandwidth | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 8.16.23 | UE Measurement Procedures / E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD | Cell1 | Cell10 | Cell30 |  |  |
| 8.16.24 | UE Measurement Procedures / E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD | Cell1 | Cell10 | Cell30 |  |  |
| 8.16.25 | UE Measurement Procedures / E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD | Cell1 | Cell10 | Cell30 |  |  |
| 8.16.26 | UE Measurement Procedures / E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD | Cell1 | Cell10 | Cell30 |  |  |
| 8.16.47 | 2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 8.16.48 | 2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 8.16.49 | 2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX | Cell1 | Cell10 |  |  |  |
| 8.16.50 | 2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX | Cell1 | Cell10 |  |  |  |
| 8.18.1 | UE Measurement Procedures / E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions | Cell1 | Cell15 |  |  |  |
| 8.19.1 | UE Measurement Procedures / E-UTRAN TDD-CDMA2000 1X event triggered reporting under fading propagation conditions | Cell1 | Cell19 |  |  |  |
| 8.20.1 | UE Measurement Procedures / RAT Measurements in CA mode / E-UTRAN FDD-FDD inter frequency event triggered reporting under fading propagation conditions in asynchronous cells | Cell6 | Cell3 Note6 | Cell13 |  | Intra-band |
|  |  | Cell6 | Cell3 | Cell10 Note4 |  | Inter-band |
| 8.20.2 | UE Measurement Procedures / RAT Measurements in CA mode / E-UTRAN TDD-TDD inter frequency event triggered reporting under fading propagation conditions in asynchronous cells | Cell6 | Cell3 Note6 | Cell13 |  | Intra-band |
|  |  | Cell6 | Cell3 | Cell10 Note4 |  | Inter-band |
| 8.20.2A | E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth | Cell6 | Cell3 Note6 | Cell13 |  | Intra-band |
|  |  | Cell6 | Cell3 | Cell10 Note4 |  | Inter-band |
| 8.20.3 | UE Measurement Procedures / RAT Measurements in CA mode / E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions | Cell1 | Cell8 | Cell4 |  | Intra-band |
|  |  | Cell1 | Cell8 | Cell10 |  | Inter-band |
| 8.20.4 | UE Measurement Procedures / RAT Measurements in CA mode / E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions | Cell1 | Cell8 | Cell4 |  | Intra-band |
|  |  | Cell1 | Cell8 | Cell10 |  | Inter-band |
| 8.20.4A | E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions for 20 MHz + 20 MHz bandwidth | Cell1 | Cell8 | Cell4 |  | Intra-band |
|  |  | Cell1 | Cell8 | Cell10 |  | Inter-band |
| 8.22.1 | UE Measurement Procedures / E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal | Cell1 | Cell2 |  |  |  |
| 8.22.2 | UE Measurement Procedures / E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal | Cell1 | Cell2 |  |  |  |
| 8.22.3 | UE Measurement Procedures / E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal | Cell6 | Cell3 |  |  |  |
| 8.22.4 | UE Measurement Procedures / E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal | Cell6 | Cell3 |  |  |  |
| 8.22.5 | UE Measurement Procedures / E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal | Cell1 | Cell2 |  |  |  |
| 8.22.6 | UE Measurement Procedures / E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal | Cell1 | Cell2 |  |  |  |
| 8.22.7 | UE Measurement Procedures / E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal | Cell6 | Cell3 |  |  |  |
| 8.22.8 | UE Measurement Procedures / E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal | Cell6 | Cell3 |  |  |  |
| 8.22.9 | UE Measurement Procedures / E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.22.10 | UE Measurement Procedures / E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.22.11 | UE Measurement Procedures / E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.22.12 | UE Measurement Procedures / E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.23.1 | E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC | Cell1 | Cell10 |  |  |  |
| 8.23.2 | E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC | Cell1 | Cell10 |  |  |  |
| 8.23.3 | E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC | Cell1 | Cell10 |  |  |  |
| 8.23.4 | E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC | Cell6 | Cell3 | Cell10 Note4 |  |  |
| 8.23.5 | E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC | Cell6 | Cell3 | Cell10 Note4 |  |  |
| 8.23.6 | E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC | Cell6 | Cell3 | Cell10 Note4 |  |  |
| 8.23.7 | E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC | Cell1 | Cell10 |  |  |  |
| 8.23.8 | E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC | Cell1 | Cell10 |  |  |  |
| 8.23.9 | E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC | Cell1 | Cell10 |  |  |  |
| 8.25.1 | E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN | Cell1 | Cell2 (WLAN) |  |  |  |
| 8.25.2 | E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN | Cell1 | Cell2 (WLAN) |  |  |  |
| 8.26.1 | E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX | Cell1 | Cell10 |  |  | Inter-Band |
| 8.26.2 | E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX | Cell1 | Cell10 |  |  | Inter-Band |
| 8.26.3 | E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX | Cell1 | Cell10 | Cell30 |  | Inter-Band |
| 8.26.4 | E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX | Cell1 | Cell10 | Cell30 |  | Inter-Band |
| 8.26.5 | E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal | Cell1 | Cell10 | Cell30 |  | Inter-Band |
| 8.26.6 | E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal | Cell1 | Cell10 | Cell30 |  | Inter-Band |
| 8.26.7 | E-UTRAN FDD-FS3 intra-frequency event triggered reporting in DRX for CRS based discovery signal | Cell1 | Cell10 | Cell30 |  | Inter-Band |
| 8.26.8 | E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal | Cell1 | Cell10 | Cell30 |  | Inter-Band |
| 8.26.9 | E-UTRAN FDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells | Cell1 | Cell10 |  |  |  |
| 8.26.10 | E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells | Cell1 | Cell10 |  |  |  |
| 9.1.1.1 | Measurement Performance Requirements / E-UTRAN / FDD Intra frequency RSRP Accuracy / Absolute | Cell1 | Cell2 |  |  |  |
| 9.1.1.1_1 | Measurement Performance Requirements / E-UTRAN / FDD Intra Frequency Absolute RSRP Accuracy (Rel-12 and forward) | Cell1 | Cell2 |  |  |  |
| 9.1.1.1_2 | Measurement Performance Requirements / E-UTRAN / FDD Intra frequency RSRP Accuracy / Absolute for UE Category 1bis | Cell1 | Cell2 |  |  |  |
| 9.1.1.1_3 | Measurement Performance Requirements / E-UTRAN / FDD Intra frequency RSRP Accuracy / Absolute for CA Idle Mode Measurements | Cell1 | Cell2 |  |  |  |
| 9.1.1.2 | Measurement Performance Requirements / E-UTRAN / FDD Intra frequency RSRP Accuracy / Relative | Cell1 | Cell2 |  |  |  |
| 9.1.1.2_2 | Measurement Performance Requirements / E-UTRAN / FDD Intra frequency RSRP Accuracy / Relative for UE Category 1bis | Cell1 | Cell2 |  |  |  |
| 9.1.2.1 | Measurement Performance Requirements / E-UTRAN / TDD Intra Frequency RSRP Accuracy / Absolute | Cell1 | Cell2 |  |  |  |
| 9.1.2.1_1 | Measurement Performance Requirements / E-UTRAN / TDD Intra Frequency Absolute RSRP Accuracy (Rel-12 and forward) | Cell1 | Cell2 |  |  |  |
| 9.1.2.1_2 | Measurement Performance Requirements / E-UTRAN / TDD Intra Frequency RSRP Accuracy / Absolute for UE Category 1bis | Cell1 | Cell2 |  |  |  |
| 9.1.2.2 | Measurement Performance Requirements / E-UTRAN / TDD Intra Frequency RSRP Accuracy / Relative | Cell1 | Cell2 |  |  |  |
| 9.1.2.2_2 | Measurement Performance Requirements / E-UTRAN / TDD Intra Frequency RSRP Accuracy / Relative for UE Category 1bis | Cell1 | Cell2 |  |  |  |
| 9.1.3.1 | Measurement Performance Requirements / E-UTRAN / FDD Inter frequency RSRP Accuracy / Absolute | Cell6 | Cell3 |  |  |  |
| 9.1.3.1_1 | Measurement Performance Requirements / E-UTRAN / FDD - FDD Inter Frequency Absolute RSRP Accuracy (Rel-12 and forward) | Cell6 | Cell3 |  |  |  |
| 9.1.3.1_2 | Measurement Performance Requirements / E-UTRAN / FDD Inter frequency RSRP Accuracy / Absolute for UE Category 1bis | Cell6 | Cell3 |  |  |  |
| 9.1.3.1_3 | Measurement Performance Requirements / E-UTRAN / FDD Inter frequency RSRP Accuracy / Absolute for CA Idle Mode Measurements for Overlapping Carrier | Cell6 | Cell3 |  |  |  |
| 9.1.3.1_4 | Measurement Performance Requirements / E-UTRAN / FDD Inter frequency RSRP Accuracy / Absolute for CA Idle Mode Measurements for Non-Overlapping Carrier | Cell6 | Cell3 |  |  |  |
| 9.1.3.2 | Measurement Performance Requirements / E-UTRAN / FDD Inter frequency RSRP Accuracy / Relative | Cell6 | Cell3 |  |  |  |
| 9.1.3.2_1 | Measurement Performance Requirements / E-UTRAN / FDD Inter frequency RSRP Accuracy / Relative (Rel-12 and forward) | Cell6 | Cell3 |  |  |  |
| 9.1.3.2_2 | Measurement Performance Requirements / E-UTRAN / FDD Inter frequency RSRP Accuracy / Relative for UE Category 1bis | Cell6 | Cell3 |  |  |  |
| 9.1.4.1 | Measurement Performance Requirements / E-UTRAN / TDD Inter Frequency RSRP Accuracy / Absolute | Cell6 | Cell3 |  |  |  |
| 9.1.4.1_1 | Measurement Performance Requirements / E-UTRAN / TDD - TDD Inter Frequency Absolute RSRP Accuracy (Rel-12 and forward) | Cell6 | Cell3 |  |  |  |
| 9.1.4.1_2 | Measurement Performance Requirements / E-UTRAN / TDD Inter Frequency RSRP Accuracy / Absolute for UE Category 1bis | Cell6 | Cell3 |  |  |  |
| 9.1.4.2 | Measurement Performance Requirements / E-UTRAN / TDD Inter Frequency RSRP Accuracy / Relative | Cell6 | Cell3 |  |  |  |
| 9.1.4.2_1 | Measurement Performance Requirements / E-UTRAN / TDD Inter Frequency RSRP Accuracy / Relative (Rel-12 and forward) | Cell6 | Cell3 |  |  |  |
| 9.1.4.2_2 | Measurement Performance Requirements / E-UTRAN / TDD Inter Frequency RSRP Accuracy / Relative for UE Category 1bis | Cell6 | Cell3 |  |  |  |
| 9.1.5.1 | Measurement Performance Requirements / E-UTRAN / FDD - TDD Inter frequency RSRP Accuracy / Absolute | Cell1 | Cell10 |  |  |  |
| 9.1.5.1_1 | Measurement Performance Requirements / E-UTRAN / FDD - TDD Inter Frequency Absolute RSRP Accuracy (Rel-12 and forward) | Cell1 | Cell10 |  |  |  |
| 9.1.5.1_2 | Measurement Performance Requirements / E-UTRAN / FDD - TDD Inter frequency RSRP Accuracy / Absolute for UE Category 1bis | Cell1 | Cell10 |  |  |  |
| 9.1.5.2 | Measurement Performance Requirements / E-UTRAN / FDD-TDD Inter frequency RSRP Accuracy / Relative | Cell1 | Cell10 |  |  |  |
| 9.1.5.2_1 | Measurement Performance Requirements / E-UTRAN / FDD-TDD Inter frequency RSRP Accuracy / Relative (Rel-12 and forward) | Cell1 | Cell10 |  |  |  |
| 9.1.5.2_2 | Measurement Performance Requirements / E-UTRAN / FDD-TDD Inter frequency RSRP Accuracy / Relative for UE Category 1bis | Cell1 | Cell10 |  |  |  |
| 9.1.6.1 | Measurement Performance Requirements / FDD Absolute RSRP Accuracy E-UTRA for Carrier Aggregation | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.6.1_1 | Measurement Performance Requirements / FDD Absolute RSRP Accuracy E-UTRA for Carrier Aggregation (Rel 12 and forward) | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.6.2 | Measurement Performance Requirements / FDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.6.2_1 | Measurement Performance Requirements / FDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation (Rel-12 and forward) | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.7.1 | Measurement Performance Requirements / TDD Absolute RSRP Accuracy E-UTRA for Carrier Aggregation | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.7.1_1 | Measurement Performance Requirements / TDD Absolute RSRP Accuracy E-UTRA for Carrier Aggregation (Rel-12 and forward) | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.7.2 | Measurement Performance Requirements / TDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.7.2_1 | Measurement Performance Requirements / TDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation (Rel-12 and forward) | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.8.1 | Measurement Performance Requirements / FDD Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) | Cell1 | Cell2 |  |  |  |
| 9.1.8.1_1 | Measurement Performance Requirements / FDD Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) (Rel-12 and forward) | Cell1 | Cell2 |  |  |  |
| 9.1.8.2 | Measurement Performance Requirements / FDD Relative RSRP Accuracy under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) | Cell1 | Cell2 |  |  |  |
| 9.1.9.1 | Measurement Performance Requirements / TDD Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) | Cell1 | Cell2 |  |  |  |
| 9.1.9.1_1 | Measurement Performance Requirements / TDD Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) (Rel-12 and forward) | Cell1 | Cell2 |  |  |  |
| 9.1.9.2 | Measurement Performance Requirements / TDD Relative RSRP Accuracy under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) | Cell1 | Cell2 |  |  |  |
| 9.1.10.1 | Measurement Performance Requirements / FDD Absolute RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Note 3) | Cell1 | Cell6 |  |  |  |
| 9.1.10.1_1 | Measurement Performance Requirements / FDD Absolute RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Rel-12 and forward) (Note 3) | Cell1 | Cell6 |  |  |  |
| 9.1.10.2 | Measurement Performance Requirements / FDD Relative RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Note 3) | Cell1 | Cell6 |  |  |  |
| 9.1.11.1 | Measurement Performance Requirements / TDD Absolute RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Note 3) | Cell1 | Cell6 |  |  |  |
| 9.1.11.1_1 | Measurement Performance Requirements / TDD Absolute RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Rel-12 and forward) (Note 3) | Cell1 | Cell6 |  |  |  |
| 9.1.11.2 | Measurement Performance Requirements / TDD Relative RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Note 3) | Cell1 | Cell6 |  |  |  |
| 9.1.12.1 | Measurement Performance Requirements / FDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.12.1_1 | Measurement Performance Requirements / FDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz (Rel 12 and forward) | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.12.2 | Measurement Performance Requirements / FDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.13.1 | Measurement Performance Requirements / TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.13.1_1 | Measurement Performance Requirements / TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz (Rel-12 and forward) | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.13.2 | Measurement Performance Requirements / TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.14.1 | FDD Intra Frequency Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) (Note 3) | Cell1 | Cell2 | Cell6 |  |  |
| 9.1.14.1_1 | FDD Intra Frequency Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) (Rel-12 and forward) (Note 3) | Cell1 | Cell2 | Cell6 |  |  |
| 9.1.14.2 | FDD Intra Frequency Relative RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) (Note 3) | Cell1 | Cell2 | Cell6 |  |  |
| 9.1.15.1 | TDD Intra Frequency Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) (Note 3) | Cell1 | Cell2 | Cell6 |  |  |
| 9.1.15.1_1 | TDD Intra Frequency Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) (Note 3) | Cell1 | Cell2 | Cell6 |  |  |
| 9.1.15.2 | TDD Intra Frequency Relative RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) (Note 3) | Cell1 | Cell2 | Cell6 |  |  |
| 9.1.16.1 | Measurement Performance Requirements / E-UTRAN / FDD Intra Frequency Absolute RSRP Accuracy for 5MHz Bandwidth | Cell1 | Cell2 |  |  |  |
| 9.1.16.1_1 | Measurement Performance Requirements / E-UTRAN / FDD Intra Frequency Absolute RSRP Accuracy for 5MHz Bandwidth (Rel-12 and forward) | Cell1 | Cell2 |  |  |  |
| 9.1.16.2 | Measurement Performance Requirements / E-UTRAN / FDD Intra Frequency Relative Accuracy of RSRP for 5MHz Bandwidth | Cell1 | Cell2 |  |  |  |
| 9.1.17.1 | Measurement Performance Requirements / E-UTRAN / FDD Inter frequency RSRP Accuracy / Absolute for 5 MHz | Cell1 | Cell10 |  |  |  |
| 9.1.17.1_1 | Measurement Performance Requirements / E-UTRAN / FDD - FDD Inter Frequency Absolute RSRP Accuracy for 5MHz Bandwidth (Rel-12 and forward) | Cell1 | Cell10 |  |  |  |
| 9.1.17.2 | Measurement Performance Requirements / E-UTRAN / FDD - FDD Inter Frequency Relative Accuracy of RSRP for 5MHz Bandwidth | Cell1 | Cell10 |  |  |  |
| 9.1.17.2_1 | Measurement Performance Requirements / E-UTRAN / FDD - FDD Inter Frequency Relative Accuracy of RSRP for 5MHz Bandwidth (Rel-12 and forward) | Cell1 | Cell10 |  |  |  |
| 9.1.18.1 | Measurement Performance Requirements / FDD Absolute RSRP Accuracy for E-UTRA for Carrier Aggregation for 10MHz + 5MHz | Cell6 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.18.1_1 | Measurement Performance Requirements / FDD Absolute RSRP Accuracy for E-UTRA for Carrier Aggregation for 10MHz + 5MHz (Rel 12 and forward) | Cell6 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.18.2 | Measurement Performance Requirements / FDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation for 10MHz + 5MHz | Cell6 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.19.1 | Measurement Performance Requirements / TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 10MHz + 5MHz | Cell6 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.19.1_1 | TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 10MHz + 5MHz (Rel 12 and forward) | Cell6 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.19.2 | Measurement Performance Requirements / TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 10MHz + 5MHz | Cell6 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.20.1 | Measurement Performance Requirements / FDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 5MHz + 5MHz bandwidth | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.20.1_1 | Measurement Performance Requirements / FDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 5MHz + 5MHz bandwidth (Rel 12 and forward) | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.20.2 | Measurement Performance Requirements / FDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 5MHz + 5MHz bandwidth | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.21.1 | Measurement Performance Requirements / TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 5MHz + 5MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.21.1_1 | Measurement Performance Requirements / TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 5MHz + 5MHz (Rel 12 and forward) | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.21.2 | Measurement Performance Requirements / TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 5MHz + 5MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.22 | FDD-TDD RSRP Accuracy E-UTRA for Carrier Aggregation with PCell in FDD | Cell1 | Cell10 | Cell30 |  |  |
| 9.1.23 | FDD-TDD RSRP Accuracy E-UTRA for Carrier Aggregation with PCell in TDD | Cell1 | Cell10 | Cell30 |  |  |
| 9.1.24.1_1 | TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz + 10MHz (Rel 12 and forward) | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.25 | Measurement Performance Requirements / FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal | Cell1 | Cell2 |  |  |  |
| 9.1.26 | Measurement Performance Requirements / TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal | Cell1 | Cell2 |  |  |  |
| 9.1.27 | Measurement Performance Requirements / FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal | Cell6 | Cell3 |  |  |  |
| 9.1.28 | Measurement Performance Requirements / TDD—TDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal | Cell6 | Cell3 |  |  |  |
| 9.1.29 | FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal | Cell1 | Cell2 |  |  |  |
| 9.1.30 | TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal | Cell1 | Cell2 |  |  |  |
| 9.1.31 | Measurement Performance Requirements / FDD-FDD inter frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal | Cell6 | Cell3 |  |  |  |
| 9.1.32 | Measurement Performance Requirements / TDD-TDD inter frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal | Cell6 | Cell3 |  |  |  |
| 9.1.33 | Measurement Performance Requirements / FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.34 | Measurement Performance Requirements / TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.35 | Measurement Performance Requirements / FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.36 | Measurement Performance Requirements / TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.41.1 | FD-FDD Intra Frequency Absolute RSRP Accuracy for UE category 0 | Cell1 | Cell2 |  |  |  |
| 9.1.41.2 | FD-FDD Intra Frequency Relative RSRP Accuracy for UE category 0 | Cell1 | Cell2 |  |  |  |
| 9.1.42.1 | HD-FDD Intra Frequency Absolute RSRP Accuracy for UE category 0 | Cell1 | Cell2 |  |  |  |
| 9.1.42.2 | HD-FDD Intra Frequency Relative RSRP Accuracy for UE category 0 | Cell1 | Cell2 |  |  |  |
| 9.1.43.1 | TDD Intra Frequency Absolute RSRP Accuracy for UE category 0 | Cell1 | Cell2 |  |  |  |
| 9.1.43.2 | TDD Intra Frequency Relative RSRP Accuracy for UE category 0 | Cell1 | Cell2 |  |  |  |
| 9.1.52 | FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | Cell1 | Cell2 |  |  |  |
| 9.1.53 | HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | Cell1 | Cell2 |  |  |  |
| 9.1.54 | TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | Cell1 | Cell2 |  |  |  |
| 9.1.55 | FDD intra frequency absolute and relative RSRP accuracies for SCell with frame structure 3 | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.57 | FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 9.1.58 | HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 9.1.59 | TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 9.1.60 | FS3 absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal with FDD PCell | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.61 | FS3 absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal with TDD PCell | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.1.1 | Measurement Performance Requirements / E-UTRAN / FDD Intra frequency RSRQ Accuracy / Absolute | Cell1 | Cell2 |  |  |  |
| 9.2.1.1_2 | Measurement Performance Requirements / E-UTRAN / FDD Intra frequency RSRQ Accuracy / Absolute for UE Category 1bis | Cell1 | Cell2 |  |  |  |
| 9.2.1.1_3 | Measurement Performance Requirements / E-UTRAN / FDD Intra frequency RSRQ Accuracy / Absolute for CA Idle Mode Measurements | Cell1 | Cell2 |  |  |  |
| 9.2.2.1 | Measurement Performance Requirements / E-UTRAN / TDD Intra Frequency RSRQ Accuracy / Absolute | Cell1 | Cell2 |  |  |  |
| 9.2.2.1_2 | Measurement Performance Requirements / E-UTRAN / TDD Intra Frequency RSRQ Accuracy / Absolute for UE Category 1bis | Cell1 | Cell2 |  |  |  |
| 9.2.3.1 | Measurement Performance Requirements / E-UTRAN / FDD Inter frequency RSRQ Accuracy / Absolute | Cell6 | Cell3 |  |  |  |
| 9.2.3.1_2 | Measurement Performance Requirements / E-UTRAN / FDD Inter frequency RSRQ Accuracy / Absolute for UE Category 1bis | Cell6 | Cell3 |  |  |  |
| 9.2.3.1_3 | Measurement Performance Requirements / E-UTRAN / FDD Inter frequency RSRQ Accuracy / Absolute for CA Idle Mode Measurements for Overlapping Carrier | Cell6 | Cell3 |  |  |  |
| 9.2.3.1_4 | Measurement Performance Requirements / E-UTRAN / FDD Inter frequency RSRQ Accuracy / Absolute for CA Idle Mode Measurements for Non-Overlapping Carrier | Cell6 | Cell3 |  |  |  |
| 9.2.3.2 | Measurement Performance Requirements / E-UTRAN / FDD Inter frequency RSRQ Accuracy / Relative | Cell6 | Cell3 |  |  |  |
| 9.2.3.2_2 | Measurement Performance Requirements / E-UTRAN / FDD Inter frequency RSRQ Accuracy / Relative for UE Category 1bis | Cell6 | Cell3 |  |  |  |
| 9.2.4.1 | Measurement Performance Requirements / E-UTRAN / TDD - TDD Inter Frequency RSRQ Accuracy / Absolute | Cell6 | Cell3 |  |  |  |
| 9.2.4.1_2 | Measurement Performance Requirements / E-UTRAN / TDD - TDD Inter Frequency RSRQ Accuracy / Absolute for UE Category 1bis | Cell6 | Cell3 |  |  |  |
| 9.2.4.2 | Measurement Performance Requirements / E-UTRAN / TDD - TDD Inter Frequency RSRQ Accuracy / Relative | Cell6 | Cell3 |  |  |  |
| 9.2.4.2_2 | Measurement Performance Requirements / E-UTRAN / TDD - TDD Inter Frequency RSRQ Accuracy / Relative for UE Category 1bis | Cell6 | Cell3 |  |  |  |
| 9.2.4A.1 | Measurement Performance Requirements / FDD - TDD Inter Frequency Absolute RSRQ Accuracy | Cell1 | Cell10 |  |  |  |
| 9.2.4A.1_2 | Measurement Performance Requirements / FDD - TDD Inter Frequency Absolute RSRQ Accuracy for UE Category 1bis | Cell1 | Cell10 |  |  |  |
| 9.2.4A.2 | Measurement Performance Requirements / FDD - TDD Inter Frequency Relative Accuracy of RSRQ | Cell1 | Cell10 |  |  |  |
| 9.2.4A.2_2 | Measurement Performance Requirements / FDD - TDD Inter Frequency Relative Accuracy of RSRQ for UE Category 1bis | Cell1 | Cell10 |  |  |  |
| 9.2.5.1 | Measurement Performance Requirements / FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.5.2 | Measurement Performance Requirements / FDD Relative RSRQ Accuracy E-UTRA for Carrier Aggregation | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.6.1 | Measurement Performance Requirements / TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.6.2 | Measurement Performance Requirements / TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.7.1 | Measurement Performance Requirements / FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) | Cell1 | Cell2 |  |  |  |
| 9.2.8.1 | Measurement Performance Requirements / TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) | Cell1 | Cell2 |  |  |  |
| 9.2.9.1 | Measurement Performance Requirements / FDD Absolute RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Note 3) | Cell1 | Cell6 |  |  |  |
| 9.2.10.1 | Measurement Performance Requirements / TDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Note 3) | Cell1 | Cell6 |  |  |  |
| 9.2.11.1 | Measurement Performance Requirements / FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.11.2 | Measurement Performance Requirements / FDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.12.1 | Measurement Performance Requirements / TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.12.2 | Measurement Performance Requirements / TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.15.1 | FDD RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) (Note 3) | Cell1 | Cell2 | Cell6 |  |  |
| 9.2.16.1 | TDD RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) (Note 3) | Cell1 | Cell2 | Cell6 |  |  |
| 9.2.17.1 | Measurement Performance Requirements / E-UTRAN / FDD Intra frequency RSRQ Accuracy / Absolute for 5MHz | Cell1 | Cell2 |  |  |  |
| 9.2.18.1 | Measurement Performance Requirements / FDD - FDD Inter Frequency Absolute RSRQ Accuracy for 5MHz Bandwidth | Cell1 | Cell10 |  |  |  |
| 9.2.18.2 | Measurement Performance Requirements / FDD - FDD Inter Frequency Relative Accuracy of RSRQ for 5MHz Bandwidth | Cell1 | Cell10 |  |  |  |
| 9.2.19.1 | Measurement Performance Requirements / FDD-FDD Inter Frequency absolute WB-RSRQ accuracy | Cell6 | Cell3 |  |  |  |
| 9.2.20.1 | Measurement Performance Requirements / TDD-TDD Inter Frequency absolute WB-RSRQ accuracy | Cell6 | Cell3 |  |  |  |
| 9.2.21.1 | Measurement Performance Requirements / FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.21.2 | Measurement Performance Requirements / FDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.22.1 | Measurement Performance Requirements / TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.22.2 | Measurement Performance Requirements / TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.23.1 | Measurement Performance Requirements / FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.23.2 | Measurement Performance Requirements / FDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.24.1 | Measurement Performance Requirements / TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.24.2 | Measurement Performance Requirements / TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.25.1 | Absolute RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD | Cell1 | Cell10 |  |  |  |
| 9.2.25.2 | Relative RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD | Cell1 | Cell10 |  |  |  |
| 9.2.26.1 | Absolute RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD | Cell1 | Cell10 |  |  |  |
| 9.2.26.2 | Relative RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD | Cell1 | Cell10 |  |  |  |
| 9.2.28 | Measurement Performance Requirements / FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal | Cell1 | Cell2 |  |  |  |
| 9.2.29 | Measurement Performance Requirements / TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal | Cell1 | Cell2 |  |  |  |
| 9.2.30 | Measurement Performance Requirements / FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal | Cell6 | Cell3 |  |  |  |
| 9.2.31 | Measurement Performance Requirements / TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal | Cell6 | Cell3 |  |  |  |
| 9.2.32 | Measurement Performance Requirements / FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.33 | Measurement Performance Requirements / TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.51 | FDD intra frequency absolute and relative RSRQ accuracies for SCell with frame structure 3 | Cell1 | Cell10 | Cell30 |  | Inter-Band |
| 9.2.52 | TDD intra frequency absolute and relative RSRQ accuracies for SCell with frame structure 3 | Cell1 | Cell10 | Cell30 |  | Inter-Band |
| 9.3.1 | Measurement Performance Requirements / E-UTRAN FDD - UTRA FDD CPICH RSCP absolute accuracy | Cell3 | Cell9 |  |  |  |
| 9.3.2 | Measurement Performance Requirements / E-UTRAN TDD- UTRA FDD CPICH RSCP absolute accuracy | Cell3 | Cell9 |  |  |  |
| 9.3.3 | Measurement Performance Requirements / E-UTRAN FDD - UTRA FDD CPICH RSCP absolute accuracy for 5MHz bandwidth | Cell3 | Cell9 |  |  |  |
| 9.4.1 | Measurement Performance Requirements / E- UTRAN FDD - UTRA FDD CPICH Ec/No absolute accuracy | Cell3 | Cell9 |  |  |  |
| 9.4.2 | Measurement Performance Requirements / E- UTRAN TDD - UTRA FDD CPICH Ec/No absolute accuracy | Cell3 | Cell9 |  |  |  |
| 9.4.3 | Measurement Performance Requirements / E-UTRAN FDD - UTRA FDD CPICH Ec/No absolute accuracy for 5MHz bandwidth | Cell3 | Cell9 |  |  |  |
| 9.5.1 | Measurement Performance Requirements / E- UTRAN FDD - UTRA TDD P-CCPCH RSCP absolute accuracy | Cell3 | Cell9 |  |  |  |
| 9.5.2 | Measurement Performance Requirements / E- UTRAN TDD - UTRA TDD P-CCPCH RSCP absolute accuracy | Cell3 | Cell9 |  |  |  |
| 9.6.1 | Measurement Performance Requirements / GSM RSSI accuracy for E-UTRAN FDD | Cell1 | Cell26 | Cell24 |  |  |
| 9.6.2 | Measurement Performance Requirements / GSM RSSI accuracy for E-UTRAN TDD | Cell1 | Cell26 | Cell24 |  |  |
| 9.9.1.1 | Measurement Performance Requirements / FDD Intra Frequency Serving Cell Absolute RSRP Accuracy | Cell1 |  |  |  |  |
| 9.9.1.1_1 | Measurement Performance Requirements / FDD Intra Frequency Serving Cell Absolute RSRP Accuracy (Rel-12 and forward) | Cell1 |  |  |  |  |
| 9.9.1.2 | Measurement Performance Requirements / FDD Intra Frequency Serving Cell Absolute RSRQ Accuracy | Cell1 |  |  |  |  |
| 9.9.2.1 | Measurement Performance Requirements / TDD Intra Frequency Serving Cell Absolute RSRP Accuracy | Cell1 |  |  |  |  |
| 9.9.2.1_1 | Measurement Performance Requirements / TDD Intra Frequency Serving Cell Absolute RSRP Accuracy (Rel-12 and forward) | Cell1 |  |  |  |  |
| 9.9.2.2 | Measurement Performance Requirements / TDD Intra Frequency Serving Cell Absolute RSRQ Accuracy | Cell1 |  |  |  |  |
| 9.11.1 | FS3 average RSSI accuracy case (PCell using FDD) | Cell1 | Cell10 |  |  | Inter-Band |
| 9.11.2 | FS3 average RSSI accuracy case (PCell using TDD) | Cell1 | Cell10 |  |  | Inter-Band |
| 9.12.1 | FS3 channel occupancy test (PCell using FDD) | Cell1 | Cell10 |  |  | Inter-Band |
| 9.12.2 | FS3 channel occupancy test (PCell using TDD) | Cell1 | Cell10 |  |  | Inter-Band |
| 12.3.1 | V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority | Cell1 |  |  |  |  |
| 12.4 | Congestion Control Measurement Test for V2X UE | Cell1 |  |  |  |  |
| 12.5 | Interruptions due to V2X Sidelink Communication | Cell1 |  |  |  |  |
| NOTE 1: Neighbour cell uses same frequency as SCC. NOTE 2: "Intra-band" notation in the table refers to both intra-band contiguous and intra-band non-contiguous CA configuration as applicable. NOTE 3: All cells are intra frequency and shall use the same range (mid) independently of the range information given in 3GPP TS 36.508 Table 4.4.2-1. NOTE 4: Low range shall be used independently on the range information given in 3GPP TS 36.508 Table 4.4.2-1. NOTE 5: The frequency range remains unchanged after any change of Cell IDs in the test procedure. NOTE 6: Mid range shall be used for intra-band non-contiguous CA independently on the range information given in 3GPP TS 36.508 Table 4.4.2-1. NOTE 7: For operating band 66 "High Range" simulated cells for all test cases shall be configured according to "High Range" as defined in 3GPP TS36.508 Table 4.3.1.1.66 when UL is not required on the cell. Otherwise "Paired High Range" shall be configured. NOTE 8: 3 Cells are randomly selected from (Cell 1, Cell 3, Cell 10, Cell 32, Cell 33, Cell 34, Cell 35, Cell 36) NOTE 9: Cell is randomly selected from (Cell 33, Cell 34, Cell 35) NOTE 10: 2 Cells are randomly selected from (Cell 1, Cell 3, Cell 10, Cell 32, Cell 36) NOTE 11: Cell is randomly selected from (Cell 5, Cell 8, Cell 9) NOTE 12: 2 Cells are randomly selected from (Cell 37, Cell 38, Cell 39) NOTE 13: 2 Cells are randomly selected from (Cell 7, Cell 37, Cell 38, Cell 39) NOTE 14: Cell is randomly selected from (Cell 37, Cell 38, Cell 39) NOTE 15: Cell in reduced performance group NOTE 16: Cell is randomly selected from (Cell 7, Cell 37, Cell 38, Cell 39) |  |  |  |  |  |  |

Table E-2: Cell configuration mapping for 3DL CA RRM testing

| Test Case | 36.521-3 Cell1 |  |  | 36.521-3 Cell2 |  |  | 36.521-3 Cell3 |  | 36.521-3 Cell4 |  | 36.521-3 Cell5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.26.3A | Default Test Settings for a CA_XA-YC Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 8.16.83(generic duplex modes) | PCC |  |  | SCC1 |  |  | SCC2 |  | Neigh |  |  |  |
| 8.16.84(generic duplex modes) | Band |  |  | Band |  |  | Band |  | Band |  |  |  |
|  | X | Cell1 |  | Y |  | Cell2 | Y | Cell3 | Y | Cell4 |  |  |
|  | Default Test Settings for a CA_XC-YA Configuration |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  | SCC1 |  |  | SCC2 |  | Neigh |  |  |  |
|  | Band |  |  | Band |  |  | Band |  | Band |  |  |  |
|  | X | Cell1 |  | X |  | Cell2 | Y | Cell3 | Y | Cell4 |  |  |
|  | Default Test Settings for a CA_XA-YA-ZA Configuration |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  | SCC1 |  |  | SCC2 |  | Neigh |  |  |  |
|  | Band |  |  | Band |  |  | Band |  | Band |  |  |  |
|  | X | Cell1 |  | Y |  | Cell2 | Z | Cell3 | Z | Cell4 |  |  |
|  | Default Test Settings for a CA_XA-XA-YA Configuration |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  | SCC1 |  |  | SCC2 |  | Neigh |  |  |  |
|  | Band |  |  | Band |  |  | Band |  | Band |  |  |  |
|  | X | Cell1 |  | X |  | Cell2 | Y | Cell3 | Y | Cell4 |  |  |
|  | Default Test Settings for a CA_XA-YA-YA Configuration |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  | SCC1 |  |  | SCC2 |  | Neigh |  |  |  |
|  | Band |  |  | Band |  |  | Band |  | Band |  |  |  |
|  | X | Cell1 |  | Y |  | Cell2 | Y | Cell3 | Y | Cell4 |  |  |
|  | Default Test Settings for a CA_XD / CA_XA-XC / CA_XC-XA Configuration |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  | SCC1 |  |  | SCC2 |  | Neigh |  |  |  |
|  | Band |  |  | Band |  |  | Band |  | Band |  |  |  |
|  | X | Cell1 |  | X |  | Cell2 | X | Cell3 | X | Cell4 |  |  |
|  | Default Test Settings for a CA_XA-YC Configuration |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  | SCC1 |  |  | Neigh |  | SCC2 |  | Neigh |  |
|  | Band |  |  | Band |  |  | Band |  | Band |  | Band |  |
| 9.1.68(generic duplex modes) Note 5 | X | Cell1 |  | Y |  | Cell2 | Y | Cell3 | Y | Cell4 | Y | Cell6 |
|  | Default Test Settings for a CA_XC-YA Configuration |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  |  |  | Neigh |  | SCC2 |  | Neigh |  |
|  | Band |  | Band |  |  |  | Band |  | Band |  | Band |  |
|  | X | Cell1 | X |  | Cell2 |  | X | Cell3 | Y | Cell4 | Y | Cell6 |
|  | Default Test Settings for a XA-YA-ZA Configuration |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  | SCC1 |  |  | Neigh |  | SCC2 |  | Neigh |  |
|  | Band |  |  | Band |  |  | Band |  | Band |  | Band |  |
|  | X | Cell1 |  | Y |  | Cell2 | Y | Cell3 | Z | Cell4 | Z | Cell6 |
|  | Default Test Settings for a XA-XA-YA Configuration |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  |  |  | Neigh |  | SCC2 |  | Neigh |  |
|  | Band |  | Band |  |  |  | Band |  | Band |  | Band |  |
|  | X | Cell1 | X |  | Cell2 |  | X | Cell3 | Y | Cell4 | Y | Cell6 |
|  | Default Test Settings for a XA-YA-YA Configuration |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  | SCC1 |  |  | Neigh |  | SCC2 |  | Neigh |  |
|  | Band |  |  | Band |  |  | Band |  | Band |  | Band |  |
|  | X | Cell1 |  | Y |  | Cell2 | Y | Cell3 | Y | Cell4 | Y | Cell6 |
|  | Default Test Settings for a CA_XD/CA_XA-XC/CA_XC-XA Configuration |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  | SCC1 |  |  | Neigh |  | SCC2 |  | Neigh |  |
|  | Band |  |  | Band |  |  | Band |  | Band |  | Band |  |
|  | X | Cell1 |  | X |  | Cell2 | X | Cell3 | X | Cell4 | X | Cell6 |
|  | Default Test Settings for a CA_XA-YC Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 8.16.85(generic duplex modes) Note 3 | PCC |  |  | SCC1 |  |  | SCC2 |  |  |  |  |  |
| 8.16.86(generic duplex modes) Note 3 9.2.55(generic duplex modes) Note 5 | Band |  |  | Band |  |  | Band |  |  |  |  |  |
|  | X | Cell1 |  | Y |  | Cell2 | Y | Cell3 |  |  |  |  |
|  | Default Test Settings for a CA_XC-YA Configuration |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  | SCC1 |  |  | SCC2 |  |  |  |  |  |
|  | Band |  |  | Band |  |  | Band |  |  |  |  |  |
|  | X | Cell1 |  | X |  | Cell2 | Y | Cell3 |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZA Configuration |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  | SCC1 |  |  | SCC2 |  |  |  |  |  |
|  | Band |  |  | Band |  |  | Band |  |  |  |  |  |
|  | X | Cell1 |  | Y |  | Cell2 | Z | Cell3 |  |  |  |  |
|  | Default Test Settings for a CA_XA-XA-YA Configuration |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  | SCC1 |  |  | SCC2 |  |  |  |  |  |
|  | Band |  |  | Band |  |  | Band |  |  |  |  |  |
| 6.2.9 | X | Cell1 |  | X |  | Cell2 | Y | Cell3 |  |  |  |  |
| 7.1.13 | Default Test Settings for a CA_XA-YA-YA Configuration |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  | SCC1 |  |  | SCC2 |  |  |  |  |  |
|  | Band |  |  | Band |  |  | Band |  |  |  |  |  |
|  | X | Cell1 |  | Y |  | Cell2 | Y | Cell3 |  |  |  |  |
|  | Default Test Settings for a CA_XD / CA_XA-XC / CA_XC-XA Configuration |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  | SCC1 |  |  | SCC2 |  |  |  |  |  |
|  | Band |  |  | Band |  |  | Band |  |  |  |  |  |
|  | X | Cell1 |  | X |  | Cell2 | X | Cell3 |  |  |  |  |
| NOTE 1: All cells shall use the same range (mid) independently of the range information given in Table 4.4.2-1 of TS 36.508 [7]. NOTE 2: X, Y and Z correspond to the different bands in the CA Configuration is specified in 5.4.2A.1 of TS 36.521-1 [10]. E.g. for CA_1A-3A-19A, X=1, Y=3, Z=19. For PCell in FDD CA Configuration, FDD bands shall precede TDD bands. E.g. for CA_13A-48A-66A, X=13, Y=66, Z=48. For PCell in TDD CA Configuration, TDD bands shall precede FDD bands. E.g. for CA_13A-48A-66A, X=48, Y=13, Z=66. NOTE 3: For intra band contiguous CA, CC addition shall be done in increasing frequency order to ensure Contiguous CC addition. NOTE 4: For simplicity, this table does not consider CA Type B. For CA Type B, the corresponding Type C default test settings shall be used. For example, for CA_XB_YA, the entry for CA_XC_YA shall be used. NOTE 5: These test cases require to swap the bands so that every band in the CA Configuration is used as PCC. In the most generic case XA-YA-ZA, the test case shall be run three times: CA_XA-YA-ZA, CA_YA-XA-ZA and CA_ZA-XA-YA. |  |  |  |  |  |  |  |  |  |  |  |  |

Table E-3: Cell configuration mapping for 2DL CA 20MHz+10MHz RRM testing

| Test Case | 36.521-3 Cell1 |  | 36.521-3 Cell2 |  | 36.521-3 Cell3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Default Test Settings for a CA_XA-YA Configuration |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  |  |
|  | Band |  | Band |  | Band |  |  |
|  | X | Cell1 | Y | Cell10 | Y | Cell30 |  |
|  | Y | Cell1 | X | Cell10 | X | Cell30 |  |
| 8.16.21 | Default Test Settings for a CA_XC Configuration |  |  |  |  |  |  |
| 8.16.22 | PCC |  | Neigh |  | SCC1 |  |  |
|  | Band |  | Band |  | Band |  |  |
|  | X | Cell1 | X | Cell2 | X | Cell4 Note1 |  |
|  | Default Test Settings for a CA_XA-XA Configuration |  |  |  |  |  |  |
|  | PCC |  | Neigh |  | SCC1 |  |  |
|  | Band |  | Band |  | Band |  |  |
|  | X | Cell1 | X | Cell2 | X | Cell4 Note1 |  |
|  | Default Test Settings for a CA_XA-YA Configuration |  |  |  |  |  |  |
|  | PCC |  | Neigh |  | SCC1 |  |  |
|  | Band |  | Band |  | Band |  |  |
|  | X | Cell6 | Y | Cell3 | Y | Cell10 Note5 |  |
|  | Y | Cell6 | X | Cell3 | X | Cell10 Note5 |  |
| 8.20.2B | Default Test Settings for a CA_XC Configuration |  |  |  |  |  |  |
|  | PCC |  | Neigh |  | SCC1 |  |  |
|  | Band |  | Band |  | Band |  |  |
|  | X | Cell6 | X | Cell3 | X | Cell13 |  |
|  | Default Test Settings for a CA_XA-XA Configuration |  |  |  |  |  |  |
|  | PCC |  | Neigh |  | SCC1 |  |  |
|  | Band |  | Band |  | Band |  |  |
|  | X | Cell6 | X | Cell3 Note6 | X | Cell13 |  |
|  | Default Test Settings for a CA_XA-YA Configuration |  |  |  |  |  |  |
|  | PCC |  | Neigh (UTRA) |  | SCC1 |  |  |
|  | Band |  | Band |  | Band |  |  |
|  | X | Cell1 | Y | Cell8 | Y | Cell10 |  |
|  | Y | Cell1 | X | Cell8 | X | Cell10 |  |
| 8.20.4B | Default Test Settings for a CA_XC Configuration |  |  |  |  |  |  |
|  | PCC |  | Neigh |  | SCC1 |  |  |
|  | Band |  | Band |  | Band |  |  |
|  | X | Cell1 | X | Cell8 | X | Cell4 |  |
|  | Default Test Settings for a CA_XA-XA Configuration |  |  |  |  |  |  |
|  | PCC |  | Neigh |  | SCC1 |  |  |
|  | Band |  | Band |  | Band |  |  |
|  | X | Cell1 | X | Cell8 | X | Cell4 |  |
|  | Default Test Settings for a CA_XA-YA Configuration |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  |  |
|  | Band |  | Band |  | Band |  |  |
| 9.1.24.1 | X | Cell1 | Y | Cell10 | Y |  | Cell30 |
| 9.1.24.2 | Y | Cell1 | X | Cell10 | X |  | Cell30 |
| 9.2.27.1 | Default Test Settings for a CA_XC Configuration |  |  |  |  |  |  |
| 9.2.27.2 | PCC |  | SCC1 |  | Neigh |  |  |
|  | Band |  | Band |  | Band |  |  |
|  | X | Cell1 | X | Cell2 | X |  | Cell4 Note1 |
|  | Default Test Settings for a CA_XA-XA Configuration |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  |  |
|  | Band |  | Band |  | Band |  |  |
|  | X | Cell1 | X | Cell2 | X |  | Cell4 Note1 |
| NOTE 1: Neighbour cell uses same frequency as SCC. NOTE 2: X, Y correspond to the different bands in the CA Configuration is specified in 5.4.2A.1 of TS 36.521-1 [10]. E.g. for CA_1A-3A, X=1, Y=3. NOTE 3: For intra-band contiguous and intra-band non-contiguous CA, 20 MHz for PCC, 10MHz for SCC are tested. NOTE 4: For inter-band CA configuration, 20 MHz for band X and 10 MHz for band Y are tested if X supports 20 MHz and Y supports 10 MHz. Otherwise, 10 MHz for band X and 20 MHz for band Y are tested. NOTE 5: Low range shall be used independently on the range information given in 3GPP TS 36.508 Table 4.4.2-1. NOTE 6: Mid range shall be used independently on the range information given in 3GPP TS 36.508 Table 4.4.2-1. NOTE 7: For simplicity, this table does not consider CA Type B. For CA Type B, the corresponding Type C default test settings shall be used. For example, for CA_XB, the entry for CA_XC shall be used. |  |  |  |  |  |  |  |

Table E-4: Ncell and Cell configuration mapping for RRM testing of Category NB1

| Test Case |  | Description |  | 36.521-3 Ncell1 |  | 36.521-3 Ncell2 |  | 36.521-3 Cell1 |  | 36.521-3 Cell2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4.2.18 |  | HD - FDD Intra frequency case for UE category NB1 In-Band mode in normal coverage |  | Ncell18 |  | Ncell5 |  | Cell1 |  | Cell11 |  |
| 4.2.19 |  | HD - FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage |  | Ncell18 |  | Ncell5 |  | Cell1 |  | Cell11 |  |
| 4.2.24 |  | HD - FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage |  | Ncell18 |  | Ncell6a |  | Cell1 |  |  |  |
| 4.2.35 |  | E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage |  | Ncell18 |  | Ncell5 |  | Cell1 |  | Cell11 |  |
| 4.2.36 |  | E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage |  | Ncell18 |  | Ncell5 |  | Cell1 |  | Cell11 |  |
| 4.2.37 |  | E-UTRAN TDD - TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage |  | Ncell18 |  | Ncell6a |  | Cell1 |  |  |  |
| 4.2.38 |  | HD - FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation |  | Ncell18 |  | Ncell5 |  | Cell1 |  | Cell11 |  |
| 6.1.15 |  | HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 under normal coverage |  | Ncell1 |  | Ncell2 |  | Cell1 |  | Cell2 |  |
| 6.1.16 |  | HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 under enhanced coverage |  | Ncell18 |  | Ncell1 |  | Cell1 |  |  |  |
| 6.1.23 |  | E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage |  | Ncell18 |  | Ncell1 |  | Cell1 |  |  |  |
| 6.1.24 |  | E-UTRAN TDD - TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage |  | Ncell18 |  | Ncell1 |  | Cell1 |  | Cell2 |  |
| 6.2.16 |  | Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage |  | Ncell18 |  |  |  | Cell1 |  |  |  |
| 6.2.17 |  | Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage |  | Ncell18 |  | Ncell1 |  | Cell1 |  |  |  |
| 6.2.18 |  | Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage |  | Ncell18 |  |  |  | Cell1 |  |  |  |
| 6.2.19 |  | TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage |  | Ncell18 |  |  |  | Cell1 |  |  |  |
| 6.2.20 |  | TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in enhanced coverage |  | Ncell18 |  | Ncell1 |  | Cell1 |  |  |  |
| 6.2.21 |  | TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage |  | Ncell18 |  |  |  | Cell1 |  |  |  |
| 7.1.17 |  | E-UTRAN HD-FDD - UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage |  | Ncell18 |  |  |  | Cell1 |  |  |  |
| 7.1.18 |  | E-UTRAN HD-FDD - UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage |  | Ncell18 |  |  |  | Cell1 |  |  |  |
| 7.1.27 |  | E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage |  | Ncell18 |  |  |  | Cell1 |  |  |  |
| 7.1.28 |  | E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage |  | Ncell18 |  |  |  | Cell1 |  |  |  |
| 7.2.9 |  | HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhance Coverage |  | Ncell1 |  |  |  |  |  |  |  |
| 7.2.15 |  | E-UTRAN TDD - TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage |  | Ncell1 |  |  |  |  |  |  |  |
| 7.3.60 |  | HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage |  | Ncell18 |  |  |  | Cell1 |  |  |  |
| 7.3.61 |  | HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in Enhanced Coverage |  | Ncell18 |  |  |  | Cell1 |  |  |  |
| 7.3.62 |  | HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage |  | Ncell18 |  |  |  | Cell1 |  |  |  |
| 7.3.63 |  | HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage |  | Ncell18 |  |  |  | Cell1 |  |  |  |
| 7.3.64 |  | HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage |  | Ncell18 |  |  |  | Cell1 |  |  |  |
| 7.3.65 |  | HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage |  | Ncell18 |  |  |  | Cell1 |  |  |  |
| 7.3.66 |  | HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage |  | Ncell18 |  |  |  | Cell1 |  |  |  |
| 7.3.67 |  | HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage |  | Ncell18 |  |  |  | Cell1 |  |  |  |
| 7.3.88 |  | TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage |  | Ncell18 |  |  |  | Cell1 |  |  |  |
| 7.3.89 |  | TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage |  | Ncell18 |  |  |  | Cell1 |  |  |  |
| 7.3.90 |  | TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage |  | Ncell18 |  |  |  | Cell1 |  |  |  |
| 7.3.91 |  | TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage |  | Ncell18 |  |  |  | Cell1 |  |  |  |
| 7.3.92 |  | TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage |  | Ncell18 |  |  |  | Cell1 |  |  |  |
| 7.3.93 |  | TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage |  | Ncell18 |  |  |  | Cell1 |  |  |  |
| 7.3.94 |  | TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage |  | Ncell18 |  |  |  | Cell1 |  |  |  |
| 7.3.95 |  | TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage |  | Ncell18 |  |  |  | Cell1 |  |  |  |
| 13.1.1.1 |  | HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage |  | Ncell18 |  | Ncell5 |  |  |  |  |  |
| 13.1.1.1_1 |  | HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage for GSO and NGSO |  | Ncell18 |  | Ncell5 |  |  |  |  |  |
| 13.1.1.2 |  | HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with serving cell RRM measurement relaxation |  | Ncell18 |  | Ncell5 |  |  |  |  |  |
| 13.1.1.2_1 |  | HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with serving cell RRM measurement relaxation for GSO and NGSO |  | Ncell18 |  | Ncell5 |  |  |  |  |  |
| 13.1.1.3 |  | HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX |  | Ncell18 |  | Ncell5 |  |  |  |  |  |
| 13.1.1.4 |  | HD – FDD Inter frequency case for UE Category NB1 Standalone mode in normal coverage |  | Ncell18 |  | Ncell5 |  |  |  |  |  |
| 13.1.1.5 |  | HD – FDD Intra frequency case for UE Category NB1 Standalone mode in enhanced coverage, location-based cell reselection for NGSO |  | Ncell18 |  | Ncell5 |  |  |  |  |  |
| 13.1.1.6 |  | HD – FDD Inter frequency case for UE Category NB1 Standalone mode in enhanced coverage, time-based cell reselection for NGSO |  | Ncell18 |  | Ncell5 |  |  |  |  |  |
| 13.1.1.3_1 |  | FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX for GSO and NGSO |  | Ncell18 |  | Ncell5 |  |  |  |  |  |
| 13.3.1.1 |  | HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage |  | Ncell1 |  | Ncell2 |  |  |  |  |  |
| 13.3.1.1_1 |  | HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage for GSO and NGSO |  | Ncell1 |  | Ncell2 |  |  |  |  |  |
| 13.3.1.2 |  | HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage |  | Ncell1 |  | Ncell2 |  |  |  |  |  |
| 13.3.1.2_1 |  | HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage for GSO and NGSO |  | Ncell1 |  | Ncell2 |  |  |  |  |  |
| 13.3.1.3 |  | HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage |  | Ncell6 |  | Ncell3 |  |  |  |  |  |
| 13.3.2.1 |  | Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in normal coverage |  | Ncell1 |  |  |  |  |  |  |  |
| 13.3.2.2 |  | Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in Enhanced Coverage |  | Ncell1 |  |  |  |  |  |  |  |
| 13.3.2.3 |  | Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs Standalone mode in Enhanced Coverage |  | Ncell1 |  |  |  |  |  |  |  |
| 13.4.1.1 |  | E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under normal coverage for Satellite Access |  | Ncell1 |  |  |  |  |  |  |  |
| 13.4.1.2 |  | E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage for Satellite Access |  | Ncell1 |  |  |  |  |  |  |  |
| 13.4.1.3 |  | E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage with segment transmission in NGSO for Satellite Access |  | Ncell1 |  |  |  |  |  |  |  |
| 13.4.2.1 |  | HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage for Satellite Access |  | Ncell1 |  |  |  |  |  |  |  |
| 13.4.2.2 |  | HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhance Coverage for Satellite Access |  | Ncell1 |  |  |  |  |  |  |  |
| 13.4.3.1 |  | HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in normal coverage |  | Ncell1 |  |  |  |  |  |  |  |
| 13.4.3.2 |  | HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in enhanced coverage |  | Ncell1 |  |  |  |  |  |  |  |
| 13.4.3.3 |  | HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 Standalone mode in Enhanced Coverage |  | Ncell1 |  |  |  |  |  |  |  |
| 13.4.3.4 |  | HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 Standalone mode in Normal Coverage |  | Ncell1 |  |  |  |  |  |  |  |
| 13.4.3.5 |  | HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage |  | Ncell1 |  |  |  |  |  |  |  |
| 13.4.3.6 |  | HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 Standalone mode in Enhanced Coverage |  | Ncell1 |  |  |  |  |  |  |  |
| 13.4.3.7 |  | HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage |  | Ncell1 |  |  |  |  |  |  |  |
| 13.4.3.8 |  | HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Enhanced Coverage |  | Ncell1 |  |  |  |  |  |  |  |
| 13.5.1 |  | HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access |  | Ncell1 |  | Ncell2 |  |  |  |  |  |
| 13.5.2 |  | HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access |  | Ncell1 |  | Ncell3 |  |  |  |  |  |
| 13.5.3 |  | HD-FDD Intra-frequency location-based neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access |  | Ncell1 |  | Ncell2 |  |  |  |  |  |
| 13.6.2.1 |  | E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage |  | Ncell1 |  |  |  |  |  |  |  |
| 13.6.2.2 |  | E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage |  | Ncell1 |  |  |  |  |  |  |  |
| 13.6.2.3 |  | E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage |  | Ncell1 |  |  |  |  |  |  |  |
| 13.6.2.4 |  | E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage |  | Ncell1 |  |  |  |  |  |  |  |
| 13.6.2.5 |  | E-UTRAN HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under normal coverage |  | Ncell1 |  |  |  |  |  |  |  |
| 13.6.2.6 |  | E-UTRAN HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under enhanced coverage |  | Ncell1 |  |  |  |  |  |  |  |

Table E-5: Cell configuration mapping for 4DL CA RRM testing

| Test Case | 36.521-3 Cell1 |  | 36.521-3 Cell2 |  | 36.521-3 Cell3 |  | 36.521-3 Cell4 |  | 36.521-3 Cell5 |  | 36.521-3 Cell6 |  | 36.521-3 Cell7 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.16.87 (generic duplex modes) 8.16.88 (generic duplex modes) | Default Test Settings for a CA_XA-YA-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | Neigh |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 | R | 6 |  |  |  |  |
|  | Default Test Settings for a CA_XA-YD |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | Neigh |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 6 |  |  |  |  |
|  | Default Test Settings for a CA_XD-YA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | Neigh |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Y | 6 |  |  |  |  |
|  | Default Test Settings for a CA_XC-YC |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | Neigh |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Y | 4 | Y | 6 |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | Neigh |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | Z | 6 |  |  |  |  |
|  | Default Test Settings for a CA_XA-YC-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | Neigh |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 6 |  |  |  |  |
|  | Default Test Settings for a CA_XC-YA-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | Neigh |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Z | 4 | Z | 6 |  |  |  |  |
|  | Default Test Settings for a CA_XE |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | Neigh |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | X | 6 |  |  |  |  |
| 9.1.69(generic duplex modes) | Default Test Settings for a CA_XA-YA-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 6 | R | 10 | R | 11 |
|  | Default Test Settings for a CA_XA-YD |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 6 | Y | 10 | Y | 11 |
|  | Default Test Settings for a CA_XD-YA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | X | 6 | Y | 10 | Y | 11 |
|  | Default Test Settings for a CA_XC-YC |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Y | 6 | Y | 10 | Y | 11 |
|  | Default Test Settings for a CA_XA-YA-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 6 | Z | 10 | Z | 11 |
|  | Default Test Settings for a CA_XA-YC-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 6 | Z | 10 | Z | 11 |
|  | Default Test Settings for a CA_XC-YA-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Y | 6 | Z | 10 | Z | 11 |
|  | Default Test Settings for a CA_XE |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | X | 6 | X | 10 | X | 11 |
| 8.16.89(generic duplex modes) Note 5 8.16.90(generic duplex modes) Note 5 | Default Test Settings for a CA_XA-YA-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YD |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XD-YA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YC |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Y | 4 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YC-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YA-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Z | 3 | Z | 4 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XE |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 |  |  |  |  |  |  |
| NOTE 1: All cells shall use the same range (mid) independently of the range information given in Table 4.4.2-1 of TS 36.508 [7]. NOTE 2: X, Y, Z and R correspond to the bands in the CA Configuration is specified in 5.4.2A.1 of TS 36.521-1 [10]. E.g. for CA_2A-2A-4A-5A, X=2, Y=2, Z=4, R=5. For PCell in FDD CA Configuration, FDD bands shall precede TDD bands. E.g. for CA_2A-2A-42A-66A, X=2, Y=2, Z=66, R=42. For PCell in TDD CA Configuration, TDD bands shall precede FDD bands. E.g. for CA_2A-2A-42A-66A, X=42, Y=2, Z=2, R=66. NOTE 3: These test cases require to swap the bands so that every band in the CA Configuration is used as PCC. In the most generic case XA-YA-ZA-RA, the test case shall be run four times: CA_XA-YA-ZA-RA, CA_YA-ZA-RA-XA, CA_ZA-RA- XA-YA and CA_RA- XA-YA-ZA. NOTE 4: For simplicity, this table does not consider CA Type B. For CA Type B, the corresponding Type C default test settings shall be used. For example, for CA_XB-YA-ZA, the entry for CA_XC-YA-ZA shall be used. NOTE 5: For intra band contiguous CA, CC addition shall be done in increasing frequency order to ensure Contiguous CC addition. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table E-6: Cell configuration mapping for 5DL CA RRM testing

| Test Case | 36.521-3 Cell1 |  | 36.521-3 Cell2 |  | 36.521-3 Cell3 |  | 36.521-3 Cell4 |  | 36.521-3 Cell5 |  | 36.521-3 Cell6 |  | 36.521-3 Cell7 |  | 36.521-3 Cell8 |  | 36.521-3 Cell9 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.16.91 (generic duplex modes) 8.16.92 (generic duplex modes) | Default Test Settings for a CA_XA-YA-ZA-RA-SA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 | S | 6 | S | 11 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 6 | Y | 11 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XE-YA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | Y | 6 | Y | 11 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Y | 4 | Y | 6 | Y | 11 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XD-YC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Y | 6 | Y | 11 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | Z | 6 | Z | 11 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YD-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Z | 6 | Z | 11 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XD-YA-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Z | 6 | Z | 11 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YC-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 6 | Z | 11 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YA-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Z | 4 | Z | 6 | Z | 11 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YC-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Y | 4 | Z | 6 | Z | 11 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZA-RC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 | R | 6 | R | 11 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZC-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | R | 6 | R | 11 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YC-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | R | 6 | R | 11 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YA-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Z | 4 | R | 6 | R | 11 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | X | 6 | X | 11 |  |  |  |  |  |  |
| 8.16.93(generic duplex modes) Note 5 8.16.94(generic duplex modes) Note 5 9.2.57(generic duplex modes) | Default Test Settings for a CA_XA-YA-ZA-RA-SA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 | S | 6 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 6 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XE-YA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | Y | 6 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Y | 4 | Y | 6 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XD-YC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Y | 6 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | Z | 6 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YD-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Z | 6 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XD-YA-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Z | 6 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YC-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 6 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YA-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Z | 4 | Z | 6 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YC-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Y | 4 | Z | 6 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZA-RC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 | R | 6 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZC-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | R | 6 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YC-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | R | 6 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YA-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Z | 4 | R | 6 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | X | 6 |  |  |  |  |  |  |  |  |
| 9.1.70(generic duplex modes) | Default Test Settings for a CA_XA-YA-ZA-RA-SA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 6 | R | 10 | R | 11 | S | 12 | S | 14 |
|  | Default Test Settings for a CA_XA-YE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 6 | Y | 10 | Y | 11 | Y | 12 | Y | 14 |
|  | Default Test Settings for a CA_XE-YA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | X | 6 | X | 10 | X | 11 | Y | 12 | Y | 14 |
|  | Default Test Settings for a CA_XC-YD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Y | 6 | Y | 10 | Y | 11 | Y | 12 | Y | 14 |
|  | Default Test Settings for a CA_XD-YC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | X | 6 | Y | 10 | Y | 11 | Y | 12 | Y | 14 |
|  | Default Test Settings for a CA_XA-YA-ZD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 6 | Z | 10 | Z | 11 | Z | 12 | Z | 14 |
|  | Default Test Settings for a CA_XA-YD-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 6 | Y | 10 | Y | 11 | Z | 12 | Z | 14 |
|  | Default Test Settings for a CA_XD-YA-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | X | 6 | Y | 10 | Y | 11 | Z | 12 | Z | 14 |
|  | Default Test Settings for a CA_XA-YC-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 6 | Z | 10 | Z | 11 | Z | 12 | Z | 14 |
|  | Default Test Settings for a CA_XC-YA-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Y | 6 | Z | 10 | Z | 11 | Z | 12 | Z | 14 |
|  | Default Test Settings for a CA_XC-YC-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Y | 6 | Y | 10 | Y | 11 | Z | 12 | Z | 14 |
|  | Default Test Settings for a CA_XA-YA-ZA-RC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 6 | R | 10 | R | 11 | R | 12 | R | 14 |
|  | Default Test Settings for a CA_XA-YA-ZC-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 6 | Z | 10 | Z | 11 | R | 12 | R | 14 |
|  | Default Test Settings for a CA_XA-YC-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 6 | Z | 10 | Z | 11 | R | 12 | R | 14 |
|  | Default Test Settings for a CA_XC-YA-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Y | 6 | Z | 10 | Z | 11 | R | 12 | R | 14 |
|  | Default Test Settings for a CA_XF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | X | 6 | X | 10 | X | 11 | X | 12 | X | 14 |
| NOTE 1: All cells shall use the same range (mid) independently of the range information given in Table 4.4.2-1 of TS 36.508 [7]. NOTE 2: X, Y, Z, R and S correspond to the bands in the CA Configuration is specified in 5.4.2A.1 of TS 36.521-1 [10]. E.g. for CA_2A-2A-4A-4A-5A, X=2, Y=2, Z=4, R=4, S=5. For PCell in FDD CA Configuration, FDD bands shall precede TDD bands. E.g. for CA_2A-2A-41A-42A-66A, X=2, Y=2, Z=66, R=41, S=42. For PCell in TDD CA Configuration, TDD bands shall precede FDD bands. E.g. for CA_2A-2A-41A-42A-66A, X=41, Y=42, Z=2, R=2, S=66. NOTE 3: These test cases require to swap the bands so that every band in the CA Configuration is used as PCC. In the most generic case CA_XA-YA-ZA-RA-SA, the test case shall be run five times: CA_XA-YA-ZA-RA-SA, CA_YA-ZA-RA-SA-XA, CA_ZA-RA-SA-XA-YA, CA_RA-SA-XA-YA-ZA and CA_SA-XA-YA-ZA-RA. NOTE 4: For simplicity, this table does not consider CA Type B. For CA Type B, the corresponding Type C default test settings shall be used. For example, for CA_XB-YA-ZA-RA, the entry for CA_XC-YA-ZA-RA shall be used. NOTE 5: For intra-band contiguous CA, CC addition shall be done in increasing frequency order to ensure Contiguous CC addition. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table E-7: Cell configuration mapping for 6DL CA RRM testing

| Test Case | 36.521-3 Cell1 |  | 36.521-3 Cell2 |  | 36.521-3 Cell3 |  | 36.521-3 Cell4 |  | 36.521-3 Cell5 |  | 36.521-3 Cell6 |  | 36.521-3 Cell7 |  | 36.521-3 Cell8 |  | 36.521-3 Cell9 |  | 36.521-3 Cell10 |  | 36.521-3 Cell11 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.16.95 8.16.96 | Default Test Settings for a CA_XA-YA-ZA-RA-SA-TA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 | S | 1a | T | 6 | T | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Y | 6 | Y | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | Z | 1a | Z | 6 | Z | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YE-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Z | 6 | Z | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XE-YA-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | Y | 1a | Z | 6 | Z | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Y | 4 | Y | 1a | Y | 6 | Y | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XE-YC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | Y | 1a | Y | 6 | Y | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZA-RD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 | R | 1a | R | 6 | R | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZD-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | Z | 1a | R | 6 | R | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YD-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Z | 1a | R | 6 | R | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YC-ZD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | Z | 6 | Z | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YD-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Z | 1a | Z | 6 | Z | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YD-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Y | 4 | Y | 1a | Z | 6 | Z | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XD-YC-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Y | 1a | Z | 6 | Z | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YC-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Y | 4 | Z | 1a | Z | 6 | Z | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZC-RC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | R | 1a | R | 6 | R | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YC-ZC-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | R | 6 | R | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZA-RC-SA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 | R | 1a | S | 6 | S | 11 |  |  |  |  |  |  |  |  |
| 8.16.97 8.16.98 9.2.58 | Default Test Settings for a CA_XA-YA-ZA-RA-SA-TA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 | S | 1a | T | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Y | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | Z | 1a | Z | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YE-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Z | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XE-YA-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | Y | 1a | Z | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Y | 4 | Y | 1a | Y | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XE-YC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | Y | 1a | Y | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZA-RD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 | R | 1a | R | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZD-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | Z | 1a | R | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YD-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Z | 1a | R | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YC-ZD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | Z | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YD-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Z | 1a | Z | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YD-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Y | 4 | Y | 1a | Z | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XD-YC-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Y | 1a | Z | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YC-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Y | 4 | Z | 1a | Z | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZC-RC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | R | 1a | R | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YC-ZC-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | R | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZA-RC-SA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 | R | 1a | S | 6 |  |  |  |  |  |  |  |  |  |  |
| 9.1.71 | Default Test Settings for a CA_XA-YA-ZA-RA-SA-TA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | R | 6 | R | 10 | S | 11 | S | 12 | T | 14 | T | 23 |
|  | Default Test Settings for a CA_XA-YF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Y | 6 | Y | 10 | Y | 11 | Y | 12 | Y | 14 | Y | 23 |
|  | Default Test Settings for a CA_XA-YA-ZE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | Z | 6 | Z | 10 | Z | 11 | Z | 12 | Z | 14 | Z | 23 |
|  | Default Test Settings for a CA_XA-YE-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1 | Y | 6 | Y | 10 | Y | 11 | Y | 12 | Z | 14 | Z | 23 |
|  | Default Test Settings for a CA_XE-YA-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | X | 1a | X | 6 | X | 10 | Y | 11 | Y | 12 | Z | 14 | Z | 23 |
|  | Default Test Settings for a CA_XC-YE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | X | 1a | Y | 6 | Y | 10 | Y | 11 | Y | 12 | Y | 14 | Y | 23 |
|  | Default Test Settings for a CA_XE-YC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | X | 1a | X | 6 | X | 10 | Y | 11 | Y | 12 | Y | 14 | Y | 23 |
|  | Default Test Settings for a CA_XA-YA-ZA-RD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | R | 6 | R | 10 | R | 11 | R | 12 | R | 14 | R | 23 |
|  | Default Test Settings for a CA_XA-YA-ZD-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | Z | 6 | Z | 10 | Z | 11 | Z | 12 | R | 14 | R | 23 |
|  | Default Test Settings for a CA_XA-YD-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Y | 6 | Y | 10 | Z | 11 | Z | 12 | R | 14 | R | 23 |
|  | Default Test Settings for a CA_XA-YC-ZD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Z | 6 | Z | 10 | Z | 11 | Z | 12 | Z | 14 | Z | 23 |
|  | Default Test Settings for a CA_XA-YD-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Y | 6 | Y | 10 | Z | 11 | Z | 12 | Z | 14 | Z | 23 |
|  | Default Test Settings for a CA_XC-YD-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Y | 1a | Y | 6 | Y | 10 | Y | 11 | Y | 12 | Z | 14 | Z | 23 |
|  | Default Test Settings for a CA_XD-YC-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | X | 1a | Y | 6 | Y | 10 | Y | 11 | Y | 12 | Z | 14 | Z | 23 |
|  | Default Test Settings for a CA_XC-YC-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Y | 1a | Y | 6 | Y | 10 | Z | 11 | Z | 12 | Z | 14 | Z | 23 |
|  | Default Test Settings for a CA_XA-YA-ZC-RC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | Z | 6 | Z | 10 | R | 11 | R | 12 | R | 14 | R | 23 |
|  | Default Test Settings for a CA_XA-YC-ZC-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Z | 6 | Z | 10 | Z | 11 | Z | 12 | R | 14 | R | 23 |
|  | Default Test Settings for a CA_XA-YA-ZA-RC-SA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | R | 6 | R | 10 | R | 11 | R | 12 | S | 14 | S | 23 |
| NOTE 1: All cells shall use the same range (mid) independently of the range information given in Table 4.4.2-1 of TS 36.508 [7]. NOTE 2: X, Y, Z, R, S and T correspond to the bands in the CA Configuration is specified in 5.4.2A.1 of TS 36.521-1 [10]. E.g. for CA_1A-3A-3A-7A-7A-8A, X=1, Y=3, Z=3, R=7, S=7, T=8. For PCell in FDD CA Configuration, FDD bands shall precede TDD bands. E.g. for CA_2A-2A-41A-42A-66A-66A, X=2, Y=2, Z=66, R=66, S=41, T=42. For PCell in TDD CA Configuration, TDD bands shall precede FDD bands. E.g. for CA_2A-2A-41A-42A-66A-66A, X=41, Y=42, Z=2, R=2, S=66, T=66. NOTE 3: For simplicity, this table does not consider CA Type B. For CA Type B, the corresponding Type C default test settings shall be used. For example, for CA_XB-YE, the entry for CA_XC-YE shall be used. NOTE 4: For intra-band contiguous CA, CC addition shall be done in increasing frequency order to ensure Contiguous CC addition. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table E-8: Cell configuration mapping for 7DL CA RRM testing

| Test Case | 36.521-3 Cell1 |  | 36.521-3 Cell2 |  | 36.521-3 Cell3 |  | 36.521-3 Cell4 |  | 36.521-3 Cell5 |  | 36.521-3 Cell6 |  | 36.521-3 Cell7 |  | 36.521-3 Cell8 |  | 36.521-3 Cell9 |  | 36.521-3 Cell10 |  | 36.521-3 Cell11 |  | 36.521-3 Cell12 |  | 36.521-3 Cell13 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.16.99 8.16.100 | Default Test Settings for a CA_XA-YA-ZA-RA-SA-TA-UA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  | Neigh |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 | S | 1a | T | 6 | T | 11 | U | 14 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  | Neigh |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | Z | 1a | Z | 6 | Z | 11 | Z | 14 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YC-ZE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  | Neigh |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | Z | 6 | Z | 11 | Z | 14 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YE-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  | Neigh |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Z | 6 | Z | 11 | Z | 14 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YE-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  | Neigh |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Y | 4 | Y | 1a | Y | 6 | Z | 11 | Z | 14 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XE-YA-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  | Neigh |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | Y | 1a | Z | 6 | Z | 11 | Z | 14 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZA-RE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  | Neigh |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 | R | 1a | R | 6 | R | 11 | R | 14 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZE-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  | Neigh |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | Z | 1a | Z | 6 | R | 11 | R | 14 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YC-ZD-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  | Neigh |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | Z | 6 | R | 11 | R | 14 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YD-ZC-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  | Neigh |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Z | 1a | Z | 6 | R | 11 | R | 14 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YE-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  | Neigh |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Z | 6 | R | 11 | R | 14 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZC-RC-SA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  | Neigh |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | R | 6 | S | 11 | S | 14 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZD-RA-SA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  | Neigh |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | Z | 1a | R | 6 | S | 11 | S | 14 |  |  |  |  |  |  |  |  |  |  |
| 8.16.102 8.16.101 9.2.59 | Default Test Settings for a CA_XA-YA-ZA-RA-SA-TA-UA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 | S | 1a | T | 6 | T | 11 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | Z | 1a | Z | 6 | Z | 11 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YC-ZE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | Z | 6 | Z | 11 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YE-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Z | 6 | Z | 11 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YE-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Y | 4 | Y | 1a | Y | 6 | Z | 11 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XE-YA-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | Y | 1a | Z | 6 | Z | 11 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZA-RE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 | R | 1a | R | 6 | R | 11 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZE-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | Z | 1a | Z | 6 | R | 11 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YC-ZD-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | Z | 6 | R | 11 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YD-ZC-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Z | 1a | Z | 6 | R | 11 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YE-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Z | 6 | R | 11 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZC-RC-SA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | R | 6 | S | 11 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZD-RA-SA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | Z | 1a | R | 6 | S | 11 |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.72 | Default Test Settings for a CA_XA-YA-ZA-RA-SA-TA-UA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  | SCC6 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | R | 6 | R | 10 | S | 11 | S | 12 | T | 13 | T | 14 | U | 23 | U | 30 |
|  | Default Test Settings for a CA_XA-YA-ZF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  | SCC6 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | Z | 6 | Z | 10 | Z | 11 | Z | 12 | Z | 13 | Z | 14 | Z | 23 | Z | 30 |
|  | Default Test Settings for a CA_XA-YC-ZE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  | SCC6 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Z | 6 | Z | 10 | Z | 11 | Z | 12 | Z | 13 | Z | 14 | Z | 23 | Z | 30 |
|  | Default Test Settings for a CA_XA-YE-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  | SCC6 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Y | 6 | Y | 10 | Y | 11 | Y | 12 | Z | 13 | Z | 14 | Z | 23 | Z | 30 |
|  | Default Test Settings for a CA_XC-YE-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  | SCC6 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Y | 1a | Y | 6 | Y | 10 | Y | 11 | Y | 12 | Y | 13 | Y | 14 | Z | 23 | Z | 30 |
|  | Default Test Settings for a CA_XE-YA-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  | SCC6 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | X | 1a | X | 6 | X | 10 | Y | 11 | Y | 12 | Z | 13 | Z | 14 | Z | 23 | Z | 30 |
|  | Default Test Settings for a CA_XA-YA-ZA-RE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  | SCC6 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | R | 6 | R | 10 | R | 11 | R | 12 | R | 13 | R | 14 | R | 23 | R | 30 |
|  | Default Test Settings for a CA_XA-YA-ZE-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  | SCC6 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | Z | 6 | Z | 10 | Z | 11 | Z | 12 | Z | 13 | Z | 14 | R | 23 | R | 30 |
|  | Default Test Settings for a CA_XA-YC-ZD-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  | SCC6 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Z | 6 | Z | 10 | Z | 11 | Z | 12 | Z | 13 | Z | 14 | R | 23 | R | 30 |
|  | Default Test Settings for a CA_XA-YD-ZC-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  | SCC6 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Y | 6 | Y | 10 | Z | 11 | Z | 12 | Z | 13 | Z | 14 | R | 23 | R | 30 |
|  | Default Test Settings for a CA_XA-YE-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  | SCC6 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Y | 6 | Y | 10 | Y | 11 | Y | 12 | Z | 13 | Z | 14 | R | 23 | R | 30 |
|  | Default Test Settings for a CA_XA-YA-ZC-RC-SA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  | SCC6 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | Z | 6 | Z | 10 | R | 11 | R | 12 | R | 13 | R | 14 | S | 23 | S | 30 |
|  | Default Test Settings for a CA_XA-YA-ZD-RA-SA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  | SCC6 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | Z | 6 | Z | 10 | Z | 11 | Z | 12 | R | 13 | R | 14 | S | 23 | S | 30 |
| NOTE 1: All cells shall use the same range (mid) independently of the range information given in Table 4.4.2-1 of TS 36.508 [7]. NOTE 2: X, Y, Z, R, S, T and U correspond to the bands in the CA Configuration is specified in 5.4.2A.1 of TS 36.521-1 [10]. E.g. for CA_1A-1A-3C-7C-28, X=1, Y=1, Z=3, R=3, S=7, T=7, U=28. For PCell in FDD CA Configuration, FDD bands shall precede TDD bands. E.g. for CA_25A-25A-41F, X=25, Y=25, Z=41, R=41, S=41, T=41, U=41. For PCell in TDD CA Configuration, TDD bands shall precede FDD bands. E.g. for CA_25A-25A-41F, X=41, Y=41, Z=41, R=41, S=41, T=25, U=25. NOTE 3: For intra-band contiguous CA, CC addition shall be done in increasing frequency order to ensure Contiguous CC addition. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Annex F (normative):
Measurement uncertainties and Test Tolerances

# F.0	General

The requirements of this clause apply to all applicable tests in the present document.

# F.1	Acceptable uncertainty of Test System (normative)

F.1.0	General

See 3GPP TS 36.521-1 [10] Annex F1.

## F.1.1	Measurement of test environments

See 3GPP TS 36.521-1 [10] Annex F1.1.

## F.1.2	Measurement of RRM requirements

Table F.1.2-1: Maximum Test System Uncertainty for RRM Requirements

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 4.2.1 E-UTRA FDD - FDD cell re-selection intra frequency | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 4.2.2 E-UTRA TDD - TDD cell re-selection intra frequency | Same as 4.2.1 |  |
| 4.2.3 E-UTRA FDD - FDD cell re-selection inter frequency | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 4.2.4 E-UTRA FDD - TDD cell re-selection inter frequency | Same as 4.2.3 |  |
| 4.2.5 E-UTRA TDD - FDD cell re-selection inter frequency | Same as 4.2.3 |  |
| 4.2.6 E-UTRA TDD - TDD cell re-selection inter frequency | Same as 4.2.3 |  |
| 4.2.7 E-UTRA FDD Inter frequency re-selection in the existence of non-allowed CSG cell | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Ês3 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês3 / Noc1 is the ratio of cell 3 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 4.2.8 E-UTRA TDD Inter frequency re-selection in the existence of non-allowed CSG cell | Same as 4.2.7 |  |
| 4.2.9 E-UTRAN FDD - FDD intra frequency cell re-selection case for 5MHz bandwidth | Same as 4.2.1 |  |
| 4.2.10 E-UTRAN FDD - FDD reselection using an increased number of carriers | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.3 dB averaged over BWConfig Noc4 ±1.0 dB averaged over BWConfig Ês4 / Noc4 ±0.3 dB averaged over BWConfig | Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN |
| 4.2.11 E-UTRAN TDD - TDD reselection using an increased number of carriers | Same as 4.2.10 | Same as 4.2.10 |
| 4.2.12 E-UTRAN FDD - FDD Intra frequency case for Cat-M1 UE in normal coverage | Same as 4.2.1 |  |
| 4.2.13 E-UTRAN HD - FDD Intra frequency case for Cat-M1 UE in normal coverage | Same as 4.2.1 |  |
| 4.2.14 E-UTRA TDD - TDD intra frequency case for Cat-M1 UE in normal coverage | Same as 4.2.1 |  |
| 4.2.15 E-UTRAN FDD - FDD Intra frequency case for Cat-M1 UE in enhanced coverage | Same as 4.2.1 |  |
| 4.2.16 E-UTRAN HD - FDD Intra frequency case for Cat-M1 UE in enhanced coverage | Same as 4.2.1 |  |
| 4.2.17 E-UTRAN TDD - TDD Intra frequency case for Cat-M1 UE in enhanced coverage | Same as 4.2.1 |  |
| 4.2.18 HD-FDD Intra frequency cell reselection for Category NB1 UE in In-Band Mode under Normal Coverage | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Êsn1 / Noc1 ±0.3 dB averaged over BWConfig Êsn2 / Noc1 ±0.3 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of eCell 1 signal / AWGN Êsn1 / Noc1 is the ratio of nCell1 signal / AWGN Êsn2 / Noc1 is the ratio of nCell2 signal / AWGN |
| 4.2.19 HD - FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Ês2 / Noc1 ±0.3 dB averaged over BWConfig Êsn1 / Noc1 ±0.3 dB averaged over BWConfig Êsn2 / Noc1 ±0.3 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of Cell 1 signal / AWGN Ês2 / Noc1 is the ratio of Cell 2 signal / AWGN Êsn1 / Noc1 is the ratio of nCell1 signal / AWGN Êsn2 / Noc1 is the ratio of nCell2 signal / AWGN |
| 4.2.20 E-UTRAN FDD - FDD Intra frequency case for UE Category 1bis | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 4.2.21 E-UTRAN TDD - TDD Intra frequency case for UE Category 1bis | Same as 4.2.20 | Same as 4.2.20 |
| 4.2.22 E-UTRAN FDD - FDD cell re-selection intra frequency case for UE configured with highSpeedEnhancedMeasFlag | Same as 4.2.1 |  |
| 4.2.23 E-UTRAN TDD - TDD cell re-selection intra frequency case for UE configured with highSpeedEnhancedMeasFlag | Same as 4.2.1 |  |
| 4.2.24 HD - FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Êsn1 / Noc1 ±0.3 dB averaged over BWConfig Êsn2 / Noc1 ±0.3 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of Cell 1 signal / AWGN Êsn1 / Noc1 is the ratio of nCell1 signal / AWGN Êsn2 / Noc1 is the ratio of nCell2 signal / AWGN |
| 4.2.28 E-UTRAN FDD - FDD Inter frequency case for CE UE in enhanced coverage | Same as 4.2.3 | Same as 4.2.3 |
| 4.2.29 E-UTRAN HD - FDD Inter frequency case for CE UE in enhanced coverage | Same as 4.2.3 | Same as 4.2.3 |
| 4.2.30 E-UTRAN TDD Inter frequency case for CE UE in enhanced coverage | Same as 4.2.3 | Same as 4.2.3 |
| 4.2.35 E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage | Same as 4.2.18 | Same as 4.2.18 |
| 4.2.36 E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage | Same as 4.2.19 | Same as 4.2.19 |
| 4.2.37 E-UTRAN TDD - TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage | Same as 4.2.24 | Same as 4.2.24 |
| 4.2.38 HD - FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation | Same as 4.2.18 | Same as 4.2.18 |
| 4.3.1.1 E-UTRA FDD - UTRAN FDD cell reselection: UTRA FDD is of higher priority | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior / Ioc ±0.3 dB CPICH Ec / Ior ±0.1 dB | Notes: Noc is the AWGN on cell 1 (E-UTRA )frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 (UTRA) frequency Ior / Ioc is the ratio of cell 2 signal / AWGN CPICH Ec / Ior is the fraction of cell 2 power assigned to the CPICH Physical channel |
| 4.3.1.2 E-UTRAN FDD - UTRAN FDD cell re-selection: UTRA FDD is of lower priority | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior / Ioc ±0.3 dB CPICH Ec / Ior ±0.1 dB | Notes: Noc is the AWGN on cell 1 (E-UTRA )frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 (UTRA) frequency Ior / Ioc is the ratio of cell 2 signal / AWGN CPICH Ec / Ior is the fraction of cell 2 power assigned to the CPICH Physical channel |
| 4.3.1.3 EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.6 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior / Ioc ±0.3 dB CPICH Ec / Ior ±0.1 dB | Notes: Noc is the AWGN on cell 1 (E-UTRA )frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB Ioc is the AWGN on cell 2 (UTRA) frequency Ior / Ioc is the ratio of cell 2 signal / AWGN CPICH Ec / Ior is the fraction of cell 2 power assigned to the CPICH Physical channel |
| 4.3.1.4 E-UTRAN FDD - UTRAN FDD cell re-selection: UTRA FDD is of lower priority for 5MHz bandwidth | Same as 4.3.1.2 |  |
| 4.3.1.5 Idle mode FDD to UTRA FDD interRAT reselection | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cells Ioc ±0.7 dB Ior / Ioc ±0.3 dB CPICH Ec / Ior ±0.1 dB | Notes: E-UTRA Cell 1: Noc is the AWGN on cell 1 (E-UTRA )frequency Ês / Noc is the ratio of cell 1 signal / AWGN UTRA Cells 2 and 3: Ioc is the AWGN on cell 2 and cell 3 (UTRA) frequency Ior / Ioc is the ratio of cell 2 and cell 3 signal / AWGN CPICH Ec / Ior is the fraction of cell 2 and cell 3 power assigned to the CPICH Physical channel |
| 4.3.2 E-UTRA FDD - UTRAN TDD cell re-selection | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Îor / Ioc ±0.3 dB PCCPCH Ec/Ior ±0.1 dB DwPCH_Ec/Ior ±0.1 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 frequency Îor / Ioc is the ratio of cell 2 signal / AWGN PCCPCH Ec / Ior is the fraction of cell 2 power assigned to the PCCPCH Physical channel DwPCH_Ec/Ior is the fraction of cell 2 power assigned to the DwPCH channel |
| 4.3.2A E-UTRA FDD to UTRA TDD cell re-selection for IncMon | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cells Ioc ±0.7 dB Îor / Ioc ±0.3 dB PCCPCH Ec/Ior ±0.1 dB DwPCH_Ec/Ior ±0.1 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 and cell 3 frequency Îor / Ioc is the ratio of cell 2 and cell 3 signal / AWGN PCCPCH Ec / Ior is the fraction of cell 2 and cell 3 power assigned to the PCCPCH Physical channel DwPCH_Ec/Ior is the fraction of cell 2 and cell 3 power assigned to the DwPCH channel |
| 4.3.3 E-UTRAN TDD - UTRAN FDD cell re-selection: UTRA FDD is of lower priority | Same as 4.3.1.2 |  |
| 4.3.3A Idle mode TDD to UTRA FDD interRAT reselection | Same as 4.3.1.5 | Same as 4.3.1.5 |
| 4.3.4.1 E-UTRA TDD - UTRAN TDD cell re-selection : UTRA is of higher priority | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Îor / Ioc ±0.3 dB PCCPCH Ec/Ior ±0.1 dB DwPCH_Ec/Ior ±0.1 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 frequency Îor / Ioc is the ratio of cell 2 signal / AWGN PCCPCH Ec / Ior is the fraction of cell 2 power assigned to the PCCPCH Physical channel DwPCH_Ec/Ior is the fraction of cell 2 power assigned to the DwPCH channel |
| 4.3.4.2 E-UTRA TDD - UTRAN TDD cell re-selection : UTRA is of lower priority | Same as 4.3.2 |  |
| 4.3.4.3 EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.6 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Îor / Ioc ±0.3 dB PCCPCH Ec/Ior ±0.1 dB DwPCH_Ec/Ior ±0.1 dB | NOTE: Noc is the AWGN on cell 1 (E-UTRA) frequency Ês / Noc is the ratio of cell 1 signal / AWGN Each Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB Ioc is the AWGN on cell 2 (UTRA) frequency Îor / Ioc is the ratio of cell 2 signal / AWGN PCCPCH Ec / Ior is the fraction of cell 2 power assigned to the PCCPCH Physical channel DwPCH_Ec/Ior is the fraction of cell 2 power assigned to the DwPCH channel |
| 4.3.4.4 E-UTRA TDD to UTRA TDD cell re-selection for IncMon | Same as 4.3.2 | Same as 4.3.2A |
| 4.4.1 E-UTRAN FDD - GSM cell re-selection | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig GSM cell Signal level ±0.7 dB | Notes: Noc is the AWGN on cell 1 (E-UTRA )frequency Ês / Noc is the ratio of cell 1 signal / AWGN Cell 2 (GSM) has only the wanted signal, without AWGN |
| 4.4.2 E-UTRAN TDD - GSM cell re-selection | Same as 4.4.1 |  |
| 4.5.1.1 RRC IDLE / E-UTRAN to HRPD Cell re-selection / E-UTRAN FDD - HRPD cell re-selection: HRPD is of lower priority | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig HRPD cell Ioc ±2.0 dB Îor / Ioc ±0.7 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 (HRPD) frequency Ior / Ioc is the ratio of cell 2 signal / AWGN |
| 4.5.2.1 E-UTRAN TDD - HRPD Cell Reselection: HRPD is of Lower Priority | Same as 4.5.1.1 | Same as 4.5.1.1 |
| 5.1.1 E-UTRAN FDD-FDD Handover intra frequency case | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 5.1.2 E-UTRAN TDD-TDD Handover intra frequency case | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.3 E-UTRAN FDD-FDD Handover inter frequency case | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 5.1.3_2 E-UTRAN FDD - FDD Inter frequency handover for UE Category 1bis | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.4 E-UTRAN TDD-TDD Handover inter frequency case | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.4_2 E-UTRAN TDD - TDD Inter frequency handover for UE Category 1bis | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.5 E-UTRAN FDD-FDD inter-frequency Handover with unknown target cell | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.5_2 E-UTRAN FDD - FDD Inter frequency handover: unknown target cell for UE Category 1bis | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.6 E-UTRAN TDD-TDD inter-frequency Handover with unknown target cell | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.6_2 E-UTRAN TDD - TDD Inter frequency handover; unknown Target Cell for UE Category 1bis | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.7 E-UTRAN FDD-TDD Handover inter frequency case | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.7_2 E-UTRAN FDD - TDD Inter frequency handover for UE Category 1bis | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.8 E-UTRAN TDD-FDD Handover inter frequency case | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.8_2 E-UTRAN TDD - FDD Inter frequency handover for UE Category 1bis | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.9 E-UTRAN FDD-FDD Intra frequency handover for 5MHz bandwidth | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.10 E-UTRAN FDD-FDD Handover intra frequency handover for UE category 0 | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.11 E-UTRAN HD-FDD Handover intra frequency handover for UE category 0 | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.12 E-UTRAN TDD-TDD Handover intra frequency handover for UE category 0 | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.13 E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.14 E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.15 E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.16 E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.17 E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.18 E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.19 E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 5.1.20 E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis | Same as 5.1.19 | Same as 5.1.19 |
| 5.1.28 E-UTRAN HD-FDD inter frequency handover for CE UEs in CEModeA | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.34 E-UTRAN HD-FDD intra frequency handover for CE UEs in CEModeA without SFN acquisition | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.41 E-UTRAN FDD – FDD Intra-band Inter-frequency sync DAPS handover | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.42 E-UTRAN FDD – FDD Intra-band Inter-frequency async DAPS handover | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.43 E-UTRAN FDD – FDD Inter-band Inter-frequency sync DAPS handover | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.44 E-UTRAN FDD – FDD Inter-band Inter-frequency async DAPS handover | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.45 E-UTRAN FDD - FDD Intra frequency DAPS handover | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.46 E-UTRAN TDD - TDD Intra frequency DAPS handover | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.47 E-UTRAN FDD - FDD Intra frequency conditional handover | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.48 E-UTRAN TDD - TDD Intra frequency conditional handover | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.49 E-UTRAN FDD - FDD Inter frequency conditional handover | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.50 E-UTRAN TDD - TDD Inter frequency conditional handover | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.51 E-UTRAN FDD - TDD Inter frequency conditional handover | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.52 E-UTRAN TDD - FDD Inter frequency conditional handover | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.53 E-UTRAN TDD – TDD Intra-band Inter-frequency sync DAPS handover | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.54 E-UTRAN TDD – TDD Inter-band Inter-frequency sync DAPS handover | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.55 E-UTRAN FDD - TDD inter-band inter-frequency synchronous DAPS handover | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.56 E-UTRAN TDD - FDD inter-band inter-frequency synchronous DAPS handover | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.57 E-UTRAN FDD – TDD Inter-band Inter-frequency async DAPS handover | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.58 E-UTRAN TDD – FDD Inter-band Inter-frequency async DAPS handover | Same as 5.1.3 | Same as 5.1.3 |
| 5.2.1 E-UTRAN FDD - UTRAN FDD handover | E-UTRA cell Noc ±1.0dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior / Ioc ±0.3 dB CPICH Ec/Ior ±0.1 dB | Notes: Noc is the AWGN on cell 1 (E-UTRA )frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 (UTRA) frequency Ior / Ioc is the ratio of cell 2 signal / AWGN CPICH Ec / Ior is the fraction of cell 2 power assigned to the CPICH Physical channel |
| 5.2.2 E-UTRAN TDD - UTRAN FDD handover | Same as 5.2.1 | Same as 5.2.1 |
| 5.2.3 E-UTRAN FDD - GSM handover | E-UTRA Cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig GSM cell Signal level ±0.7 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Cell 2 (GSM) has only the wanted signal, without AWGN |
| 5.2.4 E-UTRA TDD - UTRA TDD handover | E-UTRA Cell: Noc ± 1.0 dB averaged over BWConfig Ês / Noc ± 0.3 dB averaged over BWConfig UTRA cell Ioc ± 0.7 dB Îor / Ioc ± 0.3 dB P-CCPCH_Ec / Ior ± 0.1 dB DwPCH_Ec / Ior ± 0.1 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 frequency Îor / Ioc is the ratio of cell 2 signal / AWGN P-CCPCH_Ec / Ior is the fraction of cell 2 power assigned to the P-CCPCH physical channel. DwPCH_Ec / Ior is the fraction of cell 2 power assigned to the DwPCH channel |
| 5.2.5 E-UTRA FDD - UTRA TDD handover | Same as 5.2.4 | Same as 5.2.4 |
| 5.2.6 E-UTRA TDD - GSM handover | Same as 5.2.3 | Same as 5.2.3 |
| 5.2.7 E-UTRAN FDD - UTRAN FDD handover: unknown target cell | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior/Ioc ±0.3 dB CPICH Ec/Ior ±0.1 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on Cell 2 (UTRA) frequency Ior/Ioc is the ratio of Cell 2 signal/AWGN CPICH Ec/Ior is the fraction on Cell 2 power assigned to the CPICH physical channel |
| 5.2.8 E-UTRAN FDD - GSM handover: unknown target cell | Same as 5.2.3 | Same as 5.2.3 |
| 5.2.9 E-UTRAN TDD - GSM handover: unknown target cell | Same as 5.2.3 | Same as 5.2.3 |
| 5.2.10 E-UTRAN TDD - UTRAN TDD HO test: unknown target cell | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA TDD cell Ioc ±0.7 dB Ior/Ioc ±0.3 dB PCCPCH_Ec /Ior ±0.1 dB DwPCH_Ec /Ior ±0.1 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on Cell 2 (UTRA TDD) frequency Ior/Ioc is the ratio of Cell 2 signal/AWGN PCCPCH_Ec /Ior is the fraction of Cell 2 power assigned to the PCCPCH physical channel DwPCH_Ec /Ior is the fraction of Cell 2 power assigned to the DwPCH physical channel |
| 5.2.11 E-UTRAN FDD - UTRAN FDD Handover for 5MHz Bandwidth | Same as 5.2.1 | Same as 5.2.1 |
| 5.3.1 RRC CONNECTED / Handover from E-UTRAN to non-3GPP RATs / E-UTRAN FDD - HRPD handover | E-UTRA cell Noc ±0.7 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig HRPD cell Ioc ±2.0 dB Îor / Ioc ±0.7 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 (HRPD) frequency Ior / Ioc is the ratio of cell 2 signal / AWGN |
| 5.3.5 E-UTRAN TDD - HRPD handover | Same as 5.3.1 | Same as 5.3.1 |
| 6.1.1 E-UTRAN FDD Intra-frequency RRC Re-establishment | Same as 5.1.1 | Same as 5.1.1 |
| 6.1.1_2 E-UTRAN FDD Intra-frequency RRC Re-establishment for UE Category 1bis | Same as 6.1.1 | Same as 6.1.1 |
| 6.1.2 E-UTRAN FDD Inter-frequency RRC Re-establishment | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 6.1.2_2 E-UTRAN FDD Inter-frequency RRC Re-establishment for UE Category 1bis | Same as 6.1.2 | Same as 6.1.2 |
| 6.1.3 E-UTRAN TDD Intra-frequency RRC Re-establishment | Same as 6.1.1 | Same as 6.1.1 |
| 6.1.3_2 E-UTRAN TDD Intra-frequency RRC Re-establishment for UE Category 1bis | Same as 6.1.1 | Same as 6.1.1 |
| 6.1.4 E-UTRAN TDD Inter-frequency RRC Re-establishment | Same as 6.1.3 | Same as 6.1.3 |
| 6.1.4_2 E-UTRAN TDD Inter-frequency RRC Re-establishment for UE Category 1bis | Same as 6.1.2 | Same as 6.1.2 |
| 6.1.5 E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz Bandwidth | Same as 6.1.1 | Same as 6.1.1 |
| 6.1.6 E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0 | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 6.1.7 E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0 | Same as 6.1.6 | Same as 6.1.6 |
| 6.1.8 E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0 | Same as 6.1.6 | Same as 6.1.6 |
| 6.1.9 E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | Same as 6.1.1 | Same as 6.1.1 |
| 6.1.10 E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | Same as 6.1.1 | Same as 6.1.1 |
| 6.1.11 E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | Same as 6.1.1 | Same as 6.1.1 |
| 6.1.12 E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB | Same as 6.1.1 | Same as 6.1.1 |
| 6.1.13 E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB | Same as 6.1.1 | Same as 6.1.1 |
| 6.1.14 E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB | Same as 6.1.1 | Same as 6.1.1 |
| 6.1.15 HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig Êsn1 / Noc ±0.3 dB averaged over BWConfig Êsn2 / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of eCell 1 signal / AWGN Êsn1 / Noc is the ratio of nCell 1 signal / AWGN Êsn2 / Noc is the ratio of nCell 2 signal / AWGN |
| 6.1.16 HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage | Same as 6.1.15 | Same as 6.1.15 |
| 6.1.18 E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for CE UE in CEModeA | Same as 6.1.2 | Same as 6.1.2 |
| 6.1.23 E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage | Same as 6.1.15 | Same as 6.1.15 |
| 6.1.24 E-UTRAN TDD - TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage | Same as 6.1.15 | Same as 6.1.15 |
| 6.2.1 E-UTRAN FDD - Contention Based Random Access Test | Test 1 and Test 2: Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig Uplink absolute power measurement ±1.0 dB Uplink relative power measurement ±0.7 dB ±3Ts Uplink signal transmit timing relative to downlink | NOTE: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 6.2.1_2 E-UTRAN FDD - Contention Based Random Access Test for UE Category 1bis | Same as 6.2.1 | Same as 6.2.1 |
| 6.2.2 E-UTRAN FDD - Non Contention Based Random Access Test | Same as 6.2.1 | Same as 6.2.1 |
| 6.2.2_2 E-UTRAN FDD - Non Contention Based Random Access Test for UE Category 1bis | Same as 6.2.1 | Same as 6.2.1 |
| 6.2.3 E-UTRAN TDD - Contention Based Random Access Test | Same as 6.2.1 | Same as 6.2.1 |
| 6.2.3_2 E-UTRAN TDD - Contention Based Random Access Test for UE Category 1bis | Same as 6.2.1 | Same as 6.2.1 |
| 6.2.4 E-UTRAN TDD - Non Contention Based Random Access Test | Same as 6.2.1 | Same as 6.2.1 |
| 6.2.4_2 E-UTRAN TDD - Non Contention Based Random Access Test for UE Category 1bis | Same as 6.2.1 | Same as 6.2.1 |
| 6.2.5 E-UTRAN FDD - Contention Based Random Access Test for 5MHz bandwidth | Same as 6.2.1 | Same as 6.2.1 |
| 6.2.6 E-UTRAN FDD - Non-contention Based Random Access Test for 5MHz bandwidth | Same as 6.2.1 | Same as 6.2.1 |
| 6.2.7 E-UTRAN FDD - Non-Contention Based Random Access Test for SCell in sTAG | Test 1, Test 2 and Test 3 Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Uplink absolute power measurement ±1.0 dB Uplink relative power measurement ±0.7 dB ±3Ts Uplink signal transmit timing relative to downlink | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 6.2.8 E-UTRAN TDD - Non-Contention Based Random Access Test for SCell in sTAG | Same as 6.2.7 | Same as 6.2.7 |
| 6.2.9 3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells | Test 1, Test 2 and Test 3 Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc2 ±0.3 dB averaged over BWConfig Uplink absolute power measurement ±1.0 dB Uplink relative power measurement ±0.7 dB ±3Ts Uplink signal transmit timing relative to downlink | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 6.2.10 E-UTRAN FDD - Contention based random access test for Cat-M1 UEs in normal coverage | Test 1 and Test 2: Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig Uplink absolute power measurement ±1.0 dB Uplink relative power measurement ±0.7 dB ±3Ts Uplink signal transmit timing relative to downlink | NOTE: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 6.2.11 E-UTRAN HD-FDD - Contention based random access test for Cat-M1 UEs in normal coverage | Same as 6.2.10 | Same as 6.2.10 |
| 6.2.12 E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage | Same as 6.2.10 | Same as 6.2.10 |
| 6.2.13 E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage | Test 1 and Test 2: Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig Uplink absolute power measurement ±1.0 dB Uplink relative power measurement ±0.7 dB ±3Ts Uplink signal transmit timing relative to downlink | NOTE: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 6.2.14 E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage | Same as 6.2.13 | Same as 6.2.13 |
| 6.2.15 E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage | Same as 6.2.13 | Same as 6.2.13 |
| 6.2.16 Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage | Noc ±1.0 dB averaged over NB-IoT Cell BWConfig Ês / Noc ±0.3 dB averaged over NB-IoT Cell BWConfig Uplink absolute power measurement ±1.0 dB Uplink relative power measurement ±0.7 dB ±3Ts Uplink signal transmit timing relative to downlink | NOTE: Ês / Noc is the ratio of Ncell 1 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 6.2.17 Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage | Same as 6.2.16 | Same as 6.2.16 |
| 6.2.18 Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage | Same as 6.2.16 | Same as 6.2.16 |
| 6.2.19 TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage | Same as 6.2.16 | Same as 6.2.16 |
| 6.2.20 TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in enhanced coverage | Same as 6.2.16 | Same as 6.2.16 |
| 6.2.21 TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage | Same as 6.2.17 | Same as 6.2.17 |
| 6.3.1 Redirection from E-UTRAN FDD to UTRAN FDD | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior/Ioc ±0.3 dB CPICH Ec/Ior ±0.1 dB SCH Ec/Ior ±0.1 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on Cell 2 (UTRA) frequency Ior/Ioc is the ratio of Cell 2 signal/AWGN CPICH Ec/Ior and SCH_Ec/Ior are the fractions of Cell 2 power assigned to the CPICH and SCH physical channels |
| 6.3.1_2 Redirection from E-UTRAN FDD to UTRAN FDD for UE Category 1bis | Same as 6.3.1 | Same as 6.3.1 |
| 6.3.2 Redirection from E-UTRAN TDD to UTRAN FDD | Same as 6.3.1 | Same as 6.3.1 |
| 6.3.2_2 Redirection from E-UTRAN TDD to UTRAN FDD for UE Category 1bis | Same as 6.3.1 | Same as 6.3.1 |
| 6.3.3 Redirection from E-UTRAN FDD to GERAN when System Information is provided | E-UTRA Cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig GSM cell Signal level ±0.7 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Cell 2 (GSM) has only the wanted signal, without AWGN |
| 6.3.3_2 Redirection from E-UTRAN FDD to GERAN when System Information is provided for UE Category 1bis | Same as 6.3.3 | Same as 6.3.3 |
| 6.3.4 Redirection from E-UTRAN TDD to GERAN when System Information is provided | Same as 6.3.3 | Same as 6.3.3 |
| 6.3.4_2 Redirection from E-UTRAN TDD to GERAN when System Information is provided for UE Category 1bis | Same as 6.3.3 | Same as 6.3.3 |
| 6.3.5 E-UTRA TDD RRC connection release redirection to UTRA TDD | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior/Ioc ±0.3 dB PCCPCH_Ec/Ior ±0.1 dB DwPCH_Ec/Ior ±0.1 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 frequency Îor / Ioc is the ratio of cell 2 signal / AWGN PCCPCH Ec / Ior is the fraction of cell 2 power assigned to the PCCPCH Physical channel DwPCH_Ec/Ior is the fraction of cell 2 power assigned to the DwPCH channel |
| 6.3.5_2 E-UTRA TDD RRC connection release redirection to UTRA TDD for UE Category 1bis | Same as 6.3.5 | Same as 6.3.5 |
| 6.3.6 E-UTRA FDD RRC connection release redirection to UTRA TDD | Same as 6.3.5 | Same as 6.3.5 |
| 6.3.6_2 E-UTRA FDD RRC connection release redirection to UTRA TDD for UE Category 1bis | Same as 6.3.5 | Same as 6.3.5 |
| 6.3.7 E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided | Same as 6.3.5 | Same as 6.3.5 |
| 6.3.7_2 E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided for UE Category 1bis | Same as 6.3.5 | Same as 6.3.5 |
| 6.3.8 E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided | Same as 6.3.5 | Same as 6.3.5 |
| 6.3.8_2 E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided for UE Category 1bis | Same as 6.3.5 | Same as 6.3.5 |
| 6.3.9 Redirection from E-UTRAN FDD to UTRAN FDD without System Information | Same as 6.3.1 | Same as 6.3.1 |
| 6.3.9_2 Redirection from E-UTRAN FDD to UTRAN FDD without System Information for UE Category 1bis | Same as 6.3.1 | Same as 6.3.1 |
| 6.3.10 Redirection from E-UTRAN FDD to GERAN when System Information is not provided | Same as 6.3.3 | Same as 6.3.3 |
| 6.3.10_2 Redirection from E-UTRAN FDD to GERAN when System Information is not provided for UE Category 1bis | Same as 6.3.3 | Same as 6.3.3 |
| 6.3.11 Redirection from E-UTRAN TDD to GERAN when System Information is not provided | Same as 6.3.3 | Same as 6.3.3 |
| 6.3.11_2 Redirection from E-UTRAN TDD to GERAN when System Information is not provided for UE Category 1bis | Same as 6.3.3 | Same as 6.3.3 |
| 6.3.12 E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided | Same as 6.3.1 | Same as 6.3.1 |
| 6.3.12_2 E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided for UE Category 1bis | Same as 6.3.1 | Same as 6.3.1 |
| 7.1.1 E-UTRAN FDD - UE Transmit Timing Accuracy | Noc ±3.0 dB averaged over BWConfig Ês / Noc ±0.3 dB ±3Ts Uplink signal transmit timing relative to downlink ±0.5Ts relative during UE timing adjustment | NOTE: Ês / Noc is the ratio of cell 1 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 7.1.1_2 E-UTRAN FDD - UE Transmit Timing Accuracy for UE category 1bis | Same as 7.1.1 | Same as 7.1.1 |
| 7.1.2 E-UTRAN TDD - UE Transmit Timing Accuracy | Same as 7.1.1 | Same as 7.1.1 |
| 7.1.2_2 E-UTRAN TDD - UE Transmit Timing Accuracy for UE category 1bis | Same as 7.1.1 | Same as 7.1.1 |
| 7.1.3 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for SCell | Noc1 ±3.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB Noc2 ±3.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB ±3TS Uplink signal transmit timing relative to downlink ±0.5TS relative during UE timing adjustment | NOTE: Ês / Noc is the ratio of cell signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 7.1.3_1 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for SCell (Release 12 and forward) | Same as 7.1.3 | Same as 7.1.3 |
| 7.1.4 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell | Same as 7.1.3 | Same as 7.1.3 |
| 7.1.4_1 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell (Release 12 and forward) | Same as 7.1.3 | Same as 7.1.3 |
| 7.1.4A E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz +10 MHz bandwidth | Same as 7.1.3 | Same as 7.1.3 |
| 7.1.5 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for 5MHz Bandwidth | Same as 7.1.1 | Same as 7.1.1 |
| 7.1.6 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for SCell in sTAG | Noc1 ±3.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB Noc2 ±3.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB ±3Ts Uplink signal transmit timing relative to downlink ±0.5Ts relative during UE timing adjustment | NOTE: Ês / Noc is the ratio of cell signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 7.1.7 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG | Same as 7.1.6 | Same as 7.1.6 |
| 7.1.7A E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz bandwidth | Same as 7.1.6 | Same as 7.1.6 |
| 7.1.7B E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz bandwidth | Same as 7.1.6 | Same as 7.1.6 |
| 7.1.10 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA | Same as 7.1.1 | Same as 7.1.1 |
| 7.1.11 E-UTRAN HD-FDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA | Same as 7.1.1 | Same as 7.1.1 |
| 7.1.12 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA | Same as 7.1.1 | Same as 7.1.1 |
| 7.1.13 3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 Scells | Noc1 ±3.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB Noc2 ±3.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB Noc3 ±3.0 dB averaged over BWConfig Ês3 / Noc3 ±0.3 dB ±3Ts Uplink signal transmit timing relative to downlink ±0.5Ts relative during UE timing adjustment | NOTE: Ês / Noc is the ratio of cell signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 7.1.14 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB | Same as 7.1.1 | Same as 7.1.1 |
| 7.1.15 E-UTRAN HD-FDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB | Same as 7.1.1 | Same as 7.1.1 |
| 7.1.16 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB | Same as 7.1.1 | Same as 7.1.1 |
| 7.1.17 E-UTRAN HD-FDD - UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage | Noc ±3.0 dB averaged over BWConfig Ês / Noc ±0.3 dB ±3Ts Uplink signal transmit timing relative to downlink ±0.5Ts relative during UE timing adjustment | NOTE: Ês / Noc is the ratio of Ncell 1 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 7.1.18 E-UTRAN HD-FDD - UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under enhanced coverage | Noc ±3.0 dB averaged over BWConfig Ês / Noc ±0.3 dB ±3Ts Uplink signal transmit timing relative to downlink | NOTE: Ês / Noc is the ratio of Ncell 1 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 7.1.21 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for CE UE in CEModeA | Same as 7.1.1 | Same as 7.1.1 |
| 7.1.22 E-UTRAN HD-FDD - UE Transmit Timing Accuracy Tests for CE UE in CEModeA | Same as 7.1.1 | Same as 7.1.1 |
| 7.1.23 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for CE UE in CEModeA | Same as 7.1.1 | Same as 7.1.1 |
| 7.1.24 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for CE UE in CEModeB | Same as 7.1.1 | Same as 7.1.1 |
| 7.1.27 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage | Same as 7.1.17 | Same as 7.1.17 |
| 7.1.28 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage | Same as 7.1.18 | Same as 7.1.18 |
| 7.2.1 E-UTRAN FDD - UE Timing Advance Adjustment Accuracy | Noc1 ±3.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB Timing Advance Adjustment: ±0.5Ts | NOTE: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN The timing unit TS = 1/(15000 * 2048 ) seconds, as defined in 3GPP TS.36.211 [9] |
| 7.2.1_2 E-UTRAN FDD - UE Timing Advance Adjustment Accuracy for UE category 1bis | Same as 7.2.1 | Same as 7.2.1 |
| 7.2.2 E-UTRAN TDD - UE Timing Advance Adjustment Accuracy | Same as 7.2.1 | Same as 7.2.1 |
| 7.2.2_2 E-UTRAN TDD - UE Timing Advance Adjustment Accuracy for UE category 1bis | Same as 7.2.1 | Same as 7.2.1 |
| 7.2.3 E-UTRAN FDD - UE Timing Advance Adjustment Accuracy Test for 5MHz Bandwidth | Same as 7.2.1 | Same as 7.2.1 |
| 7.2.4 E-UTRAN FDD - UE Timing Advance Adjustment Accuracy Test for SCell in sTAG | Noc1 ±3.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB Noc2 ±3.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB Timing Advance Adjustment: ±0.5Ts | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN The timing unit TS = 1/(15000 * 2048 ) seconds, as defined in 3GPP TS.36.211 [9] |
| 7.2.5 E-UTRAN TDD - UE Timing Advance Adjustment Accuracy Test for SCell in sTAG | Same as 7.2.4 | Same as 7.2.4 |
| 7.2.5A E-UTRAN TDD - UE Timing Advance Adjustment Accuracy Test for SCell in sTAG for 20MHz +20MHz bandwidth | Same as 7.2.1 | Same as 7.2.1 |
| 7.2.5B E-UTRAN TDD - UE Timing Advance Adjustment Accuracy Test for SCell in sTAG for 20MHz +10MHz bandwidth | Same as 7.2.1 | Same as 7.2.1 |
| 7.2.6 E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA | Same as 7.2.1 | Same as 7.2.1 |
| 7.2.7 E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA | Same as 7.2.1 | Same as 7.2.1 |
| 7.2.8 E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA | Same as 7.2.1 | Same as 7.2.1 |
| 7.2.9 HD-FDD UE Timing Advance Adjustment Accuracy Test for Category NB1 UE in Standalone Mode under Enhance Coverage | Same as 7.2.1 | Same as 7.2.1 |
| 7.2.10 E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB | Same as 7.2.1 | Same as 7.2.1 |
| 7.2.11 E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB | Same as 7.2.1 | Same as 7.2.1 |
| 7.2.12 E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB | Same as 7.2.1 | Same as 7.2.1 |
| 7.2.13 E-UTRAN FDD - UE Timing Advance Adjustment Accuracy Test for sTTI and Short Processing Time | Same as 7.2.1 | Same as 7.2.1 |
| 7.2.14 E-UTRAN TDD - UE Timing Advance Adjustment Accuracy Test for sTTI and Short Processing Time | Same as 7.2.1 | Same as 7.2.1 |
| 7.2.15 E-UTRAN TDD - TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage | Same as 7.2.1 | Same as 7.2.1 |
| 7.3.1 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync | ± 0.6dB (Subtest 1&2, AWGN conditions) ± 0.8dB (Subtest 3, Fading conditions, single antenna transmission) ± 0.9dB (Subtest 4, Fading conditions, two antenna transmission) | Subtests 1 & 2: Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB Subtests 3: Overall system uncertainty for fading condition comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Average signal-to-noise ratio uncertainty 2 + Signal-to-noise ratio variation2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Signal-to-noise ratio variation ±0.5 dB Fading profile power uncertainty ±0.5 dB for single Tx Subtest 4: Same calculations as for subtest 3 but with Fading profile uncertainty of ±0.7 for two Tx. |
| 7.3.1_1 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync with 4 Rx antenna ports | Same as 7.3.1 | Same as 7.3.1 |
| 7.3.2 E-UTRAN FDD Radio Link Monitoring Test for In-sync | ± 0.8dB (Subtest 1, Fading conditions, single antenna transmission) ± 0.9dB (Subtest 2, Fading conditions, two antenna transmission) | Subtest 1: See 7.3.1 subtest 3 Subtest 2: See 7.3.1 subtest 4 |
| 7.3.2_1 E-UTRAN FDD Radio Link Monitoring Test for In-sync with 4 Rx antenna ports | Same as 7.3.2 | Same as 7.3.2 |
| 7.3.3 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync | Same as 7.3.1 | Same as 7.3.1 |
| 7.3.3_1 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync with 4 Rx antenna ports | Same as 7.3.1 | Same as 7.3.1 |
| 7.3.4 E-UTRAN TDD Radio Link Monitoring Test for In-sync | Same as 7.3.2 | Same as 7.3.2 |
| 7.3.4_1 E-UTRAN TDD Radio Link Monitoring Test for In-sync with 4 Rx antenna ports | Same as 7.3.4 | Same as 7.3.4 |
| 7.3.5 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX | ±0.9dB (Subtest 1, Fading conditions, two antenna transmission) ± 0.6dB (Subtest 2, AWGN conditions) | Subtest 1: See 7.3.1, subtest 4 Subtest 2: See 7.3.1, subtest 1 |
| 7.3.5 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX with 4 Rx antenna ports | Same as 7.3.5 | Same as 7.3.5 |
| 7.3.6 E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX | ± 0.6dB (AWGN conditions) | See 7.3.1, subtest 1 |
| 7.3.6_1 E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX with 4 Rx antenna ports | Same as 7.3.6 | Same as 7.3.6 |
| 7.3.7 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX | ±0.9dB (Subtest 1, Fading conditions, two antenna transmission) ± 0.6dB (Subtest 2, AWGN conditions) | Subtest 1: See 7.3.1, subtest 4 Subtest 2: See 7.3.1, subtest 1 |
| 7.3.7_1 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX with 4 Rx antenna ports | Same as 7.3.7 | Same as 7.3.7 |
| 7.3.8 E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX | ± 0.6dB (AWGN conditions) | See 7.3.1, subtest 1 |
| 7.3.8_1 E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX with 4 Rx antenna ports | Same as 7.3.8 | Same as 7.3.8 |
| 7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non MBSFN ABS (eICIC) | Cell 1: Ês1/Noc ± 0.9 dB Cell 2: Ês2/Noc ± 0.9 dB Ês2/ Ês1 ± 1.1 dB | System uncertainties comprise a number of quantities which are selected according to their applicability in the equations below. Uncertainties are assumed to be uncorrelated, so can be root sum squared. For Ês/Noc uncertainty AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. For Ês2/Ês1 uncertainty AWGN flatness effect has been excluded, because it would cancel out for the ratio Es2/Es1. Ês1/Noc or Ês2/Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Ês2/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 2 SNR uncertainty 2 + Cell 2 Fading profile power uncertainty 2) Component uncertainties: Cell 1 SNR uncertainty (Es1 / Noc ratio before fading) ±0.3 dB Cell 2 SNR uncertainty (Es2 / Noc ratio before fading) ±0.3 dB Fading profile power uncertainty ±0.7 dB for 2 Tx AWGN flatness and signal flatness ±2.0 dB |
| 7.3.10 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non MBSFN ABS (eICIC) | Same as 7.3.9 | Same as 7.3.9 |
| 7.3.11 E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with Non MBSFN ABS (eICIC) | Cell 1: Ês1/Noc ± 0.9 dB Cell 2: Ês2/Noc ± 0.9 dB Ês2/ Ês1 ± 1.1 dB | System uncertainties comprise a number of quantities which are selected according to their applicability in the equations below. Uncertainties are assumed to be uncorrelated, so can be root sum squared. For Ês/Noc uncertainty AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. For Ês2/Ês1 uncertainty AWGN flatness effect has been excluded, because it would cancel out for the ratio Es2/Es1. Ês1/Noc or Ês2/Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Ês2/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 2 SNR uncertainty 2 + Cell 2 Fading profile power uncertainty 2) Component uncertainties: Cell 1 SNR uncertainty (Es1 / Noc ratio before fading) ±0.3 dB Cell 2 SNR uncertainty (Es2 / Noc ratio before fading) ±0.3 dB Fading profile power uncertainty ±0.7 dB for 2 Tx AWGN flatness and signal flatness ±2.0 dB |
| 7.3.12 E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with Non MBSFN ABS (eICIC) | Same as 7.3.11 | Same as 7.3.11 |
| 7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) | Same as 7.3.9 | Same as 7.3.9 |
| 7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) | Same as 7.3.9 | Same as 7.3.9 |
| 7.3.15 E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) | Same as 7.3.11 | Same as 7.3.11 |
| 7.3.16 E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) | Same as 7.3.11 | Same as 7.3.11 |
| 7.3.17 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS assistance information and Non MBSFN ABS (feICIC) | Cell 1: Ês1/Noc ± 0.9 dB Cell 2: Ês2/Noc ± 0.9 dB Cell 3: Ês3/Noc ± 0.9 dB Ês2/ Ês1 ± 1.1 dB Ês3/ Ês1 ± 1.1 dB | System uncertainties comprise a number of quantities which are selected according to their applicability in the equations below. Uncertainties are assumed to be uncorrelated, so can be root sum squared. For Ês/Noc uncertainty AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. For Ês2/Ês1 and Ês3/Ês1 uncertainty AWGN flatness effect has been excluded, because it would cancel out for the ratio Esx/Es1. Ês1/Noc, Ês2/Noc or Ês3/Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Ês2/Ês1 or Ês3/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 2 SNR uncertainty 2 + Cell 2 Fading profile power uncertainty 2) Component uncertainties: Cell 1 SNR uncertainty (Es1 / Noc ratio before fading) ±0.3 dB Cell 2 SNR uncertainty (Es2 / Noc ratio before fading) ±0.3 dB Cell 3 SNR uncertainty (Es3 / Noc ratio before fading) ±0.3 dB Fading profile power uncertainty ±0.7 dB for 2 Tx AWGN flatness and signal flatness ±2.0 dB |
| 7.3.18 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS assistance information and Non MBSFN ABS (feICIC) | Same as 7.3.17 | Same as 7.3.17 |
| 7.3.19 E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and Non MBSFN ABS (feICIC) | Same as 7.3.17 | Same as 7.3.17 |
| 7.3.20 E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and Non MBSFN ABS (feICIC) | Same as 7.3.17 | Same as 7.3.17 |
| 7.3.21 E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and MBSFN ABS (feICIC) | Same as 7.3.17 | Same as 7.3.17 |
| 7.3.22 E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and MBSFN ABS (feICIC) | Same as 7.3.17 | Same as 7.3.17 |
| 7.3.23 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth | ± 0.9dB (Subtest 4, Fading conditions, two antenna transmission) | See 7.3.1, subtest 4 |
| 7.3.23_1 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth with 4 Rx antenna ports | Same as 7.3.23 | Same as 7.3.23 |
| 7.3.24 E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth | ± 0.9dB (Subtest 2, Fading conditions, two antenna transmission) | See 7.3.2, subtest 2 |
| 7.3.24_1 E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth with 4 Rx antenna ports | Same as 7.3.24 | Same as 7.3.24 |
| 7.3.25 E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth | ±0.6dB (AWGN conditions) | See 7.3.1, subtest 1 |
| 7.3.25_1 E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth with 4 Rx antenna ports | Same as 7.3.25 | Same as 7.3.25 |
| 7.3.26 E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE category 0 | ± 0.9dB (Fading conditions, two antenna transmission) | Overall system uncertainty for fading condition comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Average signal-to-noise ratio uncertainty 2 + Signal-to-noise ratio variation2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Signal-to-noise ratio variation ±0.5 dB Fading profile power uncertainty ±0.7 dB for two Tx |
| 7.3.26_2 E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 1bis | Same as 7.3.26 | Same as 7.3.26 |
| 7.3.27 E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE category 0 | Same as 7.3.26 | Same as 7.3.26 |
| 7.3.27_2 E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 1bis | Same as 7.3.26 | Same as 7.3.26 |
| 7.3.28 E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0 | ± 0.6dB (AWGN conditions) | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 7.3.28_2 E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE Category 1bis | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.29 E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE category 0 | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.29_2 E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 1bis | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.30 E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE category 0 | Same as 7.3.26 | Same as 7.3.26 |
| 7.3.31 E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE category 0 | Same as 7.3.26 | [Same as 7.3.26 |
| 7.3.32 E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0 | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.33 E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE category 0 | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.34 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE category 0 | Same as 7.3.26 | Same as 7.3.26 |
| 7.3.34_2 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 1bis | Same as 7.3.26 | Same as 7.3.26 |
| 7.3.35 E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0 | Same as 7.3.26 | Same as 7.3.26 |
| 7.3.35_2 E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE Category 1bis | Same as 7.3.26 | Same as 7.3.26 |
| 7.3.36 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0 | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.36_2 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE Category 1bis | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.37 E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0 | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.37_2 E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category 1bis | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.38 E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC | ± 0.9dB (Fading conditions, two antenna transmission) | Overall system uncertainty for fading condition comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Average signal-to-noise ratio uncertainty 2 + Signal-to-noise ratio variation2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Signal-to-noise ratio variation ±0.5 dB Fading profile power uncertainty ±0.7 dB for two Tx |
| 7.3.38_1 E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with 4 Rx antenna ports | Same as 7.3.38 | Same as 7.3.38 |
| 7.3.39 E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC | Same as 7.3.38 | Same as 7.3.38 |
| 7.3.39_1 E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC with 4 Rx antenna ports | Same as 7.3.39 | Same as 7.3.39 |
| 7.3.40 E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC | Same as 7.3.38 | Same as 7.3.38 |
| 7.3.41 E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity | ± 0.6dB (AWGN conditions) | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 7.3.42 E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC | Same as 7.3.41 | Same as 7.3.41 |
| 7.3.43 E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity | Same as 7.3.41 | Same as 7.3.41 |
| 7.3.44 E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD | Same as 7.3.38 | Same as 7.3.38 |
| 7.3.45 E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD | Same as 7.3.38 | Same as 7.3.38 |
| 7.3.46 E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD | Same as 7.3.41 | Same as 7.3.41 |
| 7.3.47 E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD | Same as 7.3.41 | Same as 7.3.41 |
| 7.3.48 E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A | ± 0.9dB (Fading conditions, two antenna transmission) | Overall system uncertainty for fading condition comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Average signal-to-noise ratio uncertainty 2 + Signal-to-noise ratio variation2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Signal-to-noise ratio variation ±0.5 dB Fading profile power uncertainty ±0.7 dB for two Tx |
| 7.3.49 E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A | ± 0.9dB (Fading conditions, two antenna transmission) | Overall system uncertainty for fading condition comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Average signal-to-noise ratio uncertainty 2 + Signal-to-noise ratio variation2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Signal-to-noise ratio variation ±0.5 dB Fading profile power uncertainty ±0.7 dB for two Tx |
| 7.3.50: E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-Sync in DRX for UE Category M1 Configured in CEMode A | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.51 E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE Category M1 Configured in CEMode A | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.52 E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A | ± 0.9dB (Fading conditions, two antenna transmission) | Overall system uncertainty for fading condition comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Average signal-to-noise ratio uncertainty 2 + Signal-to-noise ratio variation2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Signal-to-noise ratio variation ±0.5 dB Fading profile power uncertainty ±0.7 dB for one Tx |
| 7.3.53 E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A | ± 0.9dB (Fading conditions, two antenna transmission) | Overall system uncertainty for fading condition comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Average signal-to-noise ratio uncertainty 2 + Signal-to-noise ratio variation2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Signal-to-noise ratio variation ±0.5 dB Fading profile power uncertainty ±0.7 dB for one Tx |
| 7.3.54: E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE Category M1 Configured in CEMode A | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.55: E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.56:E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A | Same as 7.3.48 | Same as 7.3.48 |
| 7.3.57:E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A | Same as 7.3.48 | Same as 7.3.48 |
| 7.3.58:E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.59:E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.60 HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.61 HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.62 HD-FDD Radio Link Monitoring Test for In-sync with DRX for Category NB1 In-Band mode in Enhanced Coverage | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.63 HD-FDD Radio Link Monitoring Test for In-sync with DRX for Category NB1 In-Band mode in Normal Coverage | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.64 HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage | ± 0.6dB (AWGN conditions) | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 7.3.65 HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage | ± 0.6dB (AWGN conditions) | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 7.3.66 HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage | Same as 7.3.64 | Same as 7.3.64 |
| 7.3.67 HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage | Same as 7.3.64 | Same as 7.3.64 |
| 7.3.69 E-UTRAN HD-FDD Early Out-of-sync reporting Test for CE UE in CEMode A | Same as 7.3.52 | Same as 7.3.52 |
| 7.3.88 TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.89 TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.90 TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.91 TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.92 TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage | Same as 7.3.64 | Same as 7.3.64 |
| 7.3.93 TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage | Same as 7.3.65 | Same as 7.3.65 |
| 7.3.94 TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage | Same as 7.3.64 | Same as 7.3.64 |
| 7.3.95 TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage | Same as 7.3.64 | Same as 7.3.64 |
| 7.4.1 E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC | ± 0.6dB (AWGN conditions) | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 7.4.2 E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC | Same as 7.4.1 | Same as 7.4.1 |
| 7.4.3 E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity | Same as 7.4.1 | Same as 7.4.1 |
| 7.5.1 E-UTRAN FDD - UE ProSe Direct Discovery Transmission Timing Accuracy Test | Noc ±3.0 dB averaged over BWConfig Ês / Noc ±0.3 dB ±3Ts Uplink signal transmit timing relative to downlink | NOTE: Ês / Noc is the ratio of cell 1 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 7.5.4 E-UTRAN FDD - UE ProSe Direct Communication Transmission Timing Accuracy Test | Noc ±3.0 dB averaged over BWConfig Ês / Noc ±0.3 dB ±3Ts Uplink signal transmit timing relative to downlink | NOTE: Ês / Noc is the ratio of cell 1 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 7.6.1 E-UTRAN FDD-TDD CA interruption at SRS carrier based switching | ± 0.6dB (AWGN conditions) | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 7.6.2 E-UTRAN TDD-TDD CA interruption at SRS carrier based switching | Same as 7.6.1 | Same as 7.6.1 |
| 8.1.1 E-UTRAN FDD-FDD intra frequency event triggered reporting under fading propagation conditions in asynchronous cells | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.6 dB averaged over BWConfig Ês2 / Noc ±0.6 dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.1.2 E-UTRAN FDD-FDD intra frequency event triggered reporting under fading propagation conditions in synchronous cells | Same as 8.1.1 | Same as 8.1.1 |
| 8.1.3 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX | Same as 8.1.1 | Same as 8.1.1 |
| 8.1.5 E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 8.1.6 E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX | Same as 8.1.5 | Same as 8.1.5 |
| 8.1.7 UE Measurement Procedures / E-UTRAN FDD-FDD Intra-frequency event-triggered reporting under time domain measurement resource restriction with non-MBSFN ABS (eICIC) | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.6 dB averaged over BWConfig Ês2 / Noc ±0.6 dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.1.8 E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.6 dB averaged over BWConfig Ês2 / Noc ±0.6 dB averaged over BWConfig Ês3 / Noc ±0.6 dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of Cell 1 signal / AWGN Ês2 / Noc is the ratio of Cell 2 signal / AWGN Ês3 / Noc is the ratio of Cell 3 signal / AWGN Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.1.9 E-UTRAN FDD-FDD intra frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz Bandwidth | Same as 8.1.1 | Same as 8.1.1 |
| 8.1.10 E-UTRAN FDD-FDD intra frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for 5MHz Bandwidth | Same as 8.1.3 | Same as 8.1.3 |
| 8.1.11 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0 | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.6 dB averaged over BWConfig Ês2 / Noc ±0.6 dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.1.11_2 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 1bis | Same as 8.1.11 | Same as 8.1.11 |
| 8.1.12 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0 | Same as 8.1.11 | Same as 8.1.11 |
| 8.1.12_2 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 1bis | Same as 8.1.11 | Same as 8.1.11 |
| 8.1.13 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0 | Same as 8.1.11 | Same as 8.1.11 |
| 8.1.13_2 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 1bis | Same as 8.1.11 | Same as 8.1.11 |
| 8.1.17 E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0 | Same as 8.1.11 | Same as 8.1.11 |
| 8.1.17_2 E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 1bis | Same as 8.1.11 | Same as 8.1.11 |
| 8.1.18 E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0 | Same as 8.1.11 | Same as 8.1.11 |
| 8.1.18_2 E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 1bis | Same as 8.1.11 | Same as 8.1.11 |
| 8.1.19 E-UTRAN FD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0 | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 8.1.19_2 E-UTRAN FD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 1bis | Same as 8.1.19 | Same as 8.1.19 |
| 8.1.20 E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0 | Same as 8.1.19 | Same as 8.1.19 |
| 8.1.20_2 E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 1bis | Same as 8.1.19 | Same as 8.1.19 |
| 8.1.21 E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0 | Same as 8.1.19 | Same as 8.1.19 |
| 8.1.22 E-UTRAN HD- FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0 | Same as 8.1.19 | Same as 8.1.19 |
| 8.1.23 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.8 dB averaged over BWConfig Ês2 / Noc ±0.8 dB dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of cell 1 signal / Fading Ês2 / Noc is the ratio of cell 2 signal / Fading Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for 2 Tx antennas |
| 8.1.24 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA | Same as 8.1.23 | Same as 8.1.23 |
| 8.1.25 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | Same as 8.1.23 | Same as 8.1.23 |
| 8.1.26 E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA | Same as 8.1.23 | Same as 8.1.23 |
| 8.1.27 E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA | Same as 8.1.23 | Same as 8.1.23 |
| 8.1.28 E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | Same as 8.1.23 | Same as 8.1.23 |
| 8.1.29 E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA | Same as 8.1.23 | Same as 8.1.23 |
| 8.1.30 E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | Same as 8.1.23 | Same as 8.1.23 |
| 8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB | Same as 8.1.33 | Same as 8.1.33 |
| 8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB | Same as 8.1.33 | Same as 8.1.33 |
| 8.1.33 E-UTRAN HD-FDD Intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.8 dB averaged over BWConfig Ês2 / Noc ±0.8 dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for 2 Tx |
| 8.1.34 E-UTRAN HD-FDD Intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB | Same as 8.1.33 | Same as 8.1.33 |
| 8.1.35 E-UTRAN TDD Intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB | Same as 8.1.33 | Same as 8.1.33 |
| 8.1.40 E-UTRAN FDD-FDD intra-frequency event triggered reporting for UE configured with highSpeedEnhancedMeasFlag in synchronous cells | AWGN absolute power, Noc ±1.0 dB Ratio of cell 1 signal /No Fading, Ês1 / Noc ±0.3 dB Ratio of cell 2 signal /No Fading, Ês2 / Noc ±0.3 dB | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of Cell 1 signal / AWGN Êsn1 / Noc1 is the ratio of nCell1 signal / AWGN Êsn2 / Noc1 is the ratio of nCell2 signal / AWGN |
| 8.1.41 E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for CE UE in CEModeA without gap | Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.6 dB averaged over BWConfig | NOTE: Ês / Noc is the ratio of cell 1 signal / AWGN Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.1.42 E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for CE UE in CEModeA without gap | Same as 8.1.41 | Same as 8.1.41 |
| 8.2.1 E-UTRAN TDD-TDD intra frequency event triggered reporting under fading propagation conditions in synchronous cells | Same as 8.1.1 | Same as 8.1.1 |
| 8.2.2 E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX | Same as 8.1.1 | Same as 8.1.1 |
| 8.2.3 E-UTRAN TDD-TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 8.2.4 E-UTRAN TDD-TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX | Same as 8.2.3 | Same as 8.2.3 |
| 8.2.5 UE Measurement Procedures / E-UTRAN TDD-TDD Intra-frequency event-triggered reporting under time domain measurement resource restriction with non-MBSFN ABS (eICIC) | Same as 8.1.7 | Same as 8.1.7 |
| 8.2.6 E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Same as 8.1.8 | Same as 8.1.8 |
| 8.2.7 E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0 | Same as 8.1.19 | Same as 8.1.19 |
| 8.2.7_2 E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 1bis | Same as 8.1.19 | Same as 8.1.19 |
| 8.2.8 E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0 | Same as 8.1.19 | Same as 8.1.19 |
| 8.2.8_2 E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 1bis | Same as 8.1.19 | Same as 8.1.19 |
| 8.2.11 E-UTRAN TDD-TDD intra-frequency event triggered reporting for UE configured with highSpeedEnhancedMeasFlag in synchronous cells | AWGN absolute power, Noc ±1.0 dB Ratio of cell 1 signal /No Fading, Ês1 / Noc ±0.3 dB Ratio of cell 2 signal /No Fading, Ês2 / Noc ±0.3 dB | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of Cell 1 signal / AWGN Êsn1 / Noc1 is the ratio of nCell1 signal / AWGN Êsn2 / Noc1 is the ratio of nCell2 signal / AWGN |
| 8.2.14 E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for CE in CEModeA without gap | Same as 8.1.41 | Same as 8.1.41 |
| 8.3.1 E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Each Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.3.1_2 E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 1bis | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Each Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.3.2 E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells | Same as 8.3.1 | Same as 8.3.1 |
| 8.3.2_2 E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells for UE category 1bis | Same as 8.3.1_2 | Same as 8.3.1_2 |
| 8.3.3 E-UTRAN FDD-FDD Inter frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 8.3.3_2 E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used for UE category 1bis | Same as 8.3.3 | Same as 8.3.3 |
| 8.3.4 E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 8.3.5 E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX | Same as 8.3.4 | Same as 8.3.4 |
| 8.3.6 E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells | Same as 8.3.4 | Same as 8.3.4 |
| 8.3.7 E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.6 dB averaged over BWConfig Noc4 ±1.0 dB averaged over BWConfig Ês4 / Noc4 ±0.6 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on cell 4 frequency Ês4 / Noc4 is the ratio of cell 4 signal / AWGN Each Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.3.8 FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, non DRX | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.3 dB averaged over BWConfig Noc4 ±1.0 dB averaged over BWConfig Ês4 / Noc4 ±0.3 dB averaged over BWConfig | Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on cell 4 frequency Ês4 / Noc4 is the ratio of cell 4 signal / AWGN |
| 8.3.9 FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX | Same as 8.3.8 | Same as 8.3.8 |
| 8.3.12 E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeA | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Each Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.3.13 E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeA | Same as 8.3.12 | Same as 8.3.12 |
| 8.3.14 E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeB | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Each Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.3.15 E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeB | Same as 8.3.14 | Same as 8.3.14 |
| 8.3.16 E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeA when DRX is used | Same as 8.3.12 | Same as 8.3.12 |
| 8.3.17 E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeA when DRX is used | Same as 8.3.12 | Same as 8.3.12 |
| 8.3.18 E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeB when DRX is used | Same as 8.3.14 | Same as 8.3.14 |
| 8.3.19 E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeB when DRX is used | Same as 8.3.14 | Same as 8.3.14 |
| 8.4.1 E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Each Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.4.1_2 E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 1bis | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Each Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.4.2 E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells | Same as 8.4.1 | Same as 8.4.1 |
| 8.4.2_2 E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells for UE category 1bis | Same as 8.4.1_2 | Same as 8.4.1_2 |
| 8.4.3 E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used | Same as 8.3.3 | Same as 8.3.3 |
| 8.4.3_2 E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used for UE category 1bis | Same as 8.3.3 | Same as 8.3.3 |
| 8.4.4 E-UTRAN TDD-TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 8.4.5 E-UTRAN TDD-TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX | Same as 8.4.4 | Same as 8.4.4 |
| 8.4.6 E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0 | Same as 8.4.1 | Same as 8.4.1 |
| 8.4.6_2 E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0 for UE category 1bis | Same as 8.4.1_2 | Same as 8.4.1_2 |
| 8.4.7 E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group | Same as 8.3.7 | Same as 8.3.7 |
| 8.4.8 TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX | Same as 8.3.8 | Same as 8.3.8 |
| 8.4.9 TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX | Same as 8.3.9 | Same as 8.3.9 |
| 8.4.12 E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeA | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Each Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.4.13 E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeB | Same as 8.4.12 | Same as 8.4.12 |
| 8.4.14 TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeA when DRX is used | Same as 8.4.12 | Same as 8.4.12 |
| 8.4.15 E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeB when DRX is used | Same as 8.4.12 | Same as 8.4.12 |
| 8.5.1 E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions | E-UTRAN cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.6 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior/Ioc ±0.6 dB CPICH Ec/Ior ±0.1 dB | NOTE: Noc is the AWGN on Cell 1 frequency Ês / Noc is the ratio of Cell 1 signal / AWGN Ês / Noc uncertainty or Ior/Ioc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB Ioc is the AWGN on Cell 2 (UTRA) frequency Ior/Ioc is the ratio of Cell 2 signal/AWGN CPICH Ec/Ior is the fraction of Cell 2 power assigned to the CPICH physical channel |
| 8.5.2 E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior/Ioc ±0.3 dB CPICH Ec/Ior ±0.1 dB SCH Ec/Ior ±0.1 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on Cell 2 (UTRA) frequency Ior/Ioc is the ratio of Cell 2 signal/AWGN CPICH Ec/Ior is the fraction of Cell 2 power assigned to the CPICH physical channel SCH Ec/Ior is the fraction of Cell 2 power assigned to the SCH physical channel |
| 8.5.3 E-UTRAN FDD - UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions | Same as 8.5.1 | Same as 8.5.1 |
| 8.5.4 E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior/Ioc ±0.3 dB CPICH Ec/Ior ±0.1 dB SCH Ec/Ior ±0.1 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on Cell 2 (UTRA) frequency Ior/Ioc is the ratio of Cell 2 signal/AWGN CPICH Ec/Ior is the fraction of Cell 2 power assigned to the CPICH physical channel SCH Ec/Ior is the fraction of Cell 2 power assigned to the SCH physical channel |
| 8.5.6 E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions | Same as 8.5.4 | Same as 8.5.4 |
| 8.5.7 E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions for 5MHz Bandwidth | Same as 8.5.1 | Same as 8.5.1 |
| 8.5.8 E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.6 dB averaged over BWConfig UTRA cells Ioc ±0.7 dB Ior/Ioc ±0.6 dB CPICH Ec/Ior ±0.1 dB SCH Ec/Ior ±0.1 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on Cells 2, 3, 4 (UTRA) frequency Ior/Ioc is the ratio of Cells 2, 3, 4 signal/AWGN CPICH Ec/Ior is the fraction of Cells 2, 3, 4 power assigned to the CPICH physical channel SCH Ec/Ior is the fraction of Cells 2, 3, 4 power assigned to the SCH physical channel |
| 8.6.1 E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions | Same as 8.5.1 | Same as 8.5.1 |
| 8.6.3 E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX | Same as 8.5.8 | Same as 8.5.8 |
| 8.7.1 E-UTRAN TDD - UTRAN TDD event triggered reporting under fading propagation conditions | E-UTRA cell Noc ±0.7 dB averaged over BWConfig Ês / Noc ±0.6 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior/Ioc ±0.6 dB PCCPCH Ec/Ior ±0.1 dB DwPCH_Ec/Ior ±0.1 dB | Notes: Noc is the AWGN on cell 1 (E-UTRA )frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ês / Noc uncertainty or Ior / Ioc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty or Ior / Ioc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB Ioc is the AWGN on cell 2 (UTRA) frequency Ior / Ioc is the ratio of cell 2 signal / AWGN PCCPCH Ec / Ior is the fraction of cell 2 power assigned to the PCCPCH Physical channel DwPCH_Ec/Ior is the fraction of cell 2 power assigned to the DwPCH channel |
| 8.7.2 E-UTRAN TDD - UTRAN TDD cell search when DRX is used under fading propagation conditions | Same as 8.7.1 | Same as 8.7.1 |
| 8.7.3 E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting under AWGN propagation conditions | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior/Ioc ±0.3 dB PCCPCH Ec/Ior ±0.1 dB DwPCH_Ec/Ior ±0.1 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 (UTRA) frequency Ior / Ioc is the ratio of cell 2 signal / AWGN PCCPCH Ec / Ior is the fraction of cell 2 power assigned to the PCCPCH Physical channel DwPCH_Ec/Ior is the fraction of cell 2 power assigned to the DwPCH channel |
| 8.7.4 E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior/Ioc ±0.3 dB PCCPCH Ec/Ior ±0.1 dB DwPCH_Ec/Ior ±0.1 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 (UTRA) frequency Ior / Ioc is the ratio of cell 2 signal / AWGN PCCPCH Ec/Ior is the fraction of cell 2 power assigned to the PCCPCH Physical channel DwPCH_Ec/Ior is the fraction of cell 2 power assigned to the DwPCH channel |
| 8.7.5 E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX | E-UTRA cell Noc ±1 dB averaged over BWConfig Ês / Noc ±0.6 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior/Ioc ±0.6 dB PCCPCH Ec/Ior ±0.1 dB DwPCH_Ec/Ior ±0.1 dB | Notes: Noc is the AWGN on cell 1 (E-UTRA) frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ês / Noc uncertainty or Ior / Ioc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty or Ior / Ioc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB Ioc is the AWGN on cells 2, 3 and 4 (UTRA) frequency Ior / Ioc is the ratio of cell 2 signal / AWGN PCCPCH Ec / Ior is the fraction of cell 2. 3 or 4 power assigned to the PCCPCH Physical channel DwPCH_Ec/Ior is the fraction of cell 2, 3 or 4 power assigned to the DwPCH channel |
| 8.7A.1 E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX | Same as 8.7.5 | Same as 8.7.5 |
| 8.8.1 E-UTRAN FDD - GSM event triggered reporting in AWGN | E-UTRA Cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig GSM cell Signal level ±0.7 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Cell 2 (GSM) has only the wanted signal, without AWGN |
| 8.8.2 E-UTRAN FDD- GSM event triggered reporting when DRX is used in AWGN | Same as 8.8.1 | Same as 8.8.1 |
| 8.9.1 E-UTRAN FDD - UTRAN TDD event triggered reporting under fading propagation conditions | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.6 dB averaged over BWConfig UTRA TDD cell Ioc ±0.7 dB Ior/Ioc ±0.6 dB PCCPCH_Ec /Ior ±0.1 dB DwPCH_Ec /Ior ±0.1 dB | Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on Cell 2 (UTRA TDD) frequency Ior/Ioc is the ratio of Cell 2 signal/AWGN PCCPCH_Ec /Ior is the fraction on Cell 2 power assigned to the CPCCPCH physical channel DwPCH_Ec /Ior is the fraction on Cell 2 power assigned to the DwPCH physical channel Ês / Noc and Ior/Ioc uncertainty for fading condition comprise two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty |
| 8.9.2 E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions | Same as 8.7.4 | Same as 8.7.4 |
| 8.10.1 E-UTRAN TDD - GSM event triggered reporting in AWGN | Same as 8.8.1 | Same as 8.8.1 |
| 8.10.2 E-UTRAN TDD- GSM event triggered reporting when DRX is used in AWGN | Same as 8.8.2 | Same as 8.8.2 |
| 8.11.1 Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.6 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Ês2 / Noc2 uncertainty or Ês3 / Noc3 uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.11.2 E-UTRAN TDD - E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions | Same as 8.11.1 | Same as 8.11.1 |
| 8.11.3 E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions | E-UTRA cells Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior/Ioc ±0.6 dB CPICH Ec/Ior ±0.1 dB | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ioc is the AWGN on Cell 3 (UTRA) frequency Ior/Ioc is the ratio of Cell 3 signal/AWGN CPICH Ec/Ior is the fraction of Cell 3 power assigned to the CPICH physical channel Ês2 / Noc2 uncertainty or Ior/Ioc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.11.4 InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search | E-UTRA cells Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior/Ioc ±0.6 dB PCCPCH Ec/Ior ±0.1 dB DwPCH_Ec/Ior ±0.1 dB | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ioc is the AWGN on cell 3 (UTRA) frequency Ior / Ioc is the ratio of cell 3 signal / AWGN PCCPCH Ec / Ior is the fraction of cell 3 power assigned to the PCCPCH Physical channel DwPCH_Ec/Ior is the fraction of cell 3 power assigned to the DwPCH channel Ês2 / Noc2 uncertainty or Ior/Ioc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc or Ior/Ioc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.11.5 Combined E-UTRAN FDD - E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions | E-UTRA cells Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig GSM cell Signal level ±0.7 dB | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Cell 3 (GSM) has only the wanted signal, without AWGN Ês1 / Noc1 uncertainty or Ês2 / Noc2 uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.11.6 Combined E-UTRAN TDD - E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions | Same as 8.11.5 | Same as 8.11.5 |
| 8.14.1 E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells | Same as 8.3.1 | Same as 8.3.1 |
| 8.14.2 E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells | Same as 8.3.1 | Same as 8.3.1 |
| 8.14.3 E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | Same as 8.4.4 | Same as 8.4.4 |
| 8.15.1 E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells | Same as 8.4.1 | Same as 8.4.1 |
| 8.15.2 E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells | Same as 8.4.1 | Same as 8.4.1 |
| 8.15.3 E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | Same as 8.4.4 | Same as 8.4.4 |
| 8.16.1 E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig Ês3 / Noc2 ±0.6 dB averaged over BWConfig Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN Ês1 / Noc1, Ês2 / Noc2 uncertainty and Ês3 / Noc2 uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.16.2 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX | Same as 8.16.1 | Same as 8.16.1 |
| 8.16.3 E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Ês3 / Noc2 ±0.3 dB averaged over BWConfig Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of Cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of Cell 2 signal / AWGN Ês3 / Noc2 is the ratio of Cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.16.4 E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX | Same as 8.16.3 | Same as 8.16.3 |
| 8.16.5 E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth | Same as 8.16.1 | Same as 8.16.1 |
| 8.16.6 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth | Same as 8.16.1 | Same as 8.16.1 |
| 8.16.7 E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth | Same as 8.16.3 | Same as 8.16.3 |
| 8.16.8 E-UTRAN TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth | Same as 8.16.3 | Same as 8.16.3 |
| 8.16.9 E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz | Same as 8.16.1 | Same as 8.16.1 |
| 8.16.10 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz | Same as 8.16.1 | Same as 8.16.1 |
| 8.16.11 E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz | Same as 8.16.3 | Same as 8.16.3 |
| 8.16.12 E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz | Same as 8.16.3 | Same as 8.16.3 |
| 8.16.13 E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz+5MHz | Same as 8.16.1 | Same as 8.16.1 |
| 8.16.14 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5MHz+5MHz | Same as 8.16.1 | Same as 8.16.1 |
| 8.16.15 E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 5MHz+5MHz | Same as 8.16.3 | Same as 8.16.3 |
| 8.16.16 E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 5MHz+5MHz | Same as 8.16.3 | Same as 8.16.3 |
| 8.16.17 E-UTRAN FDD activation and deactivation of known SCell in non-DRX | Same as 8.16.37 | Same as 8.16.37 |
| 8.16.17A E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz | Same as 8.16.17 | Same as 8.16.17 |
| 8.16.18 E-UTRAN TDD activation and deactivation of known SCell in non-DRX | Same as 8.16.17 | Same as 8.16.17 |
| 8.16.18 E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz | Same as 8.16.17 | Same as 8.16.17 |
| 8.16.21 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz bandwidth | Same as 8.16.1 | Same as 8.16.1 |
| 8.16.22 E-UTRAN TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20MHz+10MHz bandwidth | Same as 8.16.3 | Same as 8.16.3 |
| 8.16.23 E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD | Same as 8.16.1 | Same as 8.16.1 |
| 8.16.24 E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD | Same as 8.16.1 | Same as 8.16.1 |
| 8.16.25 E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD | Same as 8.16.3 | Same as 8.16.3 |
| 8.16.26 E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD | Same as 8.16.3 | Same as 8.16.3 |
| 8.16.27 E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with PCell in FDD | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.6 dB averaged over BWConfig Ês4 / Noc3 ±0.6 dB averaged over BWConfig Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Ês4 / Noc3 is the ratio of cell 4 signal / AWGN Ês2 / Noc2, Ês3 / Noc3 uncertainty and Ês4 / Noc3 uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.16.28 E-UTRAN TDD-FDD 3DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with PCell in TDD | Same as 8.16.27 | Same as 8.16.27 |
| 8.16.29, 3DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX | Same as 8.16.27 | Same as 8.16.27 |
| 8.16.30, 3DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX | Same as 8.16.27 | Same as 8.16.27 |
| 8.16.31 E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.3 dB averaged over BWConfig Ês4 / Noc3 ±0.3 dB averaged over BWConfig Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Ês4 / Noc3 is the ratio of cell 4 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.16.32 E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD | Same as 8.16.31 | Same as 8.16.31 |
| 8.16.33 E-UTRAN FDD 3DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX | Same as 8.16.31 | Same as 8.16.31 |
| 8.16.34 E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX | Same as 8.16.31 | Same as 8.16.31 |
| 8.16.35 3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.3 dB averaged over BWConfig Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.16.36 3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX | Same as 8.16.35 | Same as 8.16.35 |
| 8.16.37 3DL FDD CA Activation and Deactivation of known SCell in non-DRX | Same as 8.16.35 | Same as 8.16.35 |
| 8.16.38 3DL TDD CA Activation and Deactivation of known SCell in non-DRX | Same as 8.16.35 | Same as 8.16.35 |
| 8.16.39 E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD | Same as 8.16.35 | Same as 8.16.35 |
| 8.16.40 E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD | Same as 8.16.35 | Same as 8.16.35 |
| 8.16.41 3 DL FDD CA activation and deactivation of unknown SCell in non-DRX | Same as 8.16.35 | Same as 8.16.35 |
| 8.16.42 3 DL TDD CA activation and deactivation of unknown SCell in non-DRX | Same as 8.16.35 | Same as 8.16.35 |
| 8.16.47 2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX | Same as 8.16.17 | Same as 8.16.17 |
| 8.16.48 2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX | Same as 8.16.17 | Same as 8.16.17 |
| 8.16.49 2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX | Same as 8.16.17 | Same as 8.16.17 |
| 8.16.50 2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX | Same as 8.16.17 | Same as 8.16.17 |
| 8.16.51 E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.6 dB averaged over BWConfig Noc4 ±1.0 dB averaged over BWConfig Ês4 / Noc4 ±0.6 dB averaged over BWConfig Ês5 / Noc4 ±0.6 dB averaged over BWConfig Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN Ês5 / Noc4 is the ratio of cell 5 signal / AWGN Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4 uncertainty and Ês5 / Noc4 uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.16.52 E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX | Same as 8.16.51 | Same as 8.16.51 |
| 8.16.53 4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX | Same as 8.16.51 | Same as 8.16.51 |
| 8.16.54 4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX | Same as 8.16.51 | Same as 8.16.51 |
| 8.16.55 4DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.3 dB averaged over BWConfig Noc4 ±1.0 dB averaged over BWConfig Ês4 / Noc4 ±0.3 dB averaged over BWConfig Ês5 / Noc4 ±0.3 dB averaged over BWConfig Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN Ês5 / Noc4 is the ratio of cell 5 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.16.56 4DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX | Same as 8.16.55 | Same as 8.16.55 |
| 8.16.57 4DL FDD CA Activation and Deactivation of Known SCell in Non-DRX | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.3 dB averaged over BWConfig Noc4 ±1.0 dB averaged over BWConfig Ês4 / Noc4 ±0.3 dB averaged over BWConfig Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.16.58 4DL TDD CA Activation and Deactivation of Known SCell in Non-DRX | Same as 8.16.57 | Same as 8.16.57 |
| 8.16.59 4DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX | Same as 8.16.57 | Same as 8.16.57 |
| 8.16.60 4DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX | Same as 8.16.57 | Same as 8.16.57 |
| 8.16.61 4DL FDD CA Activation and Deactivation of Unknown SCell in Non-DRX | Same as 8.16.57 | Same as 8.16.57 |
| 8.16.62 4DL TDD CA Activation and Deactivation of Unknown SCell in Non-DRX | Same as 8.16.57 | Same as 8.16.57 |
| 8.16.63 4DL PCell in FDD CA Activation and Deactivation of Unknown SCell in Non-DRX | Same as 8.16.57 | Same as 8.16.57 |
| 8.16.64 4DL PCell in TDD CA Activation and Deactivation of Unknown SCell in Non-DRX | Same as 8.16.57 | Same as 8.16.57 |
| 8.16.65 5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX | Same as 8.16.71 | Same as 8.16.71 |
| 8.16.66 5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX | Same as 8.16.71 | Same as 8.16.71 |
| 8.16.67 5DL Pcell in FDD CA activation and deactivation of unknown SCell in non-DRX | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.3 dB averaged over BWConfig Noc4 ±1.0 dB averaged over BWConfig Ês4 / Noc4 ±0.3 dB averaged over BWConfig Noc5 ±1.0 dB averaged over BWConfig Ês5 / Noc5 ±0.3 dB averaged over BWConfig Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN Noc5 is the AWGN on frequency 5 Ês5 / Noc5 is the ratio of cell 5 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.16.68 5DL Pcell in TDD CA activation and deactivation of unknown SCell in non-DRX | Same as 8.16.67 | Same as 8.16.67 |
| 8.16.69 5DL FDD CA activation and deactivation of unknown SCell in non-DRX | Same as 8.16.67 | Same as 8.16.67 |
| 8.16.70 5DL TDD CA activation and deactivation of unknown SCell in non-DRX | Same as 8.16.67 | Same as 8.16.67 |
| 8.16.71 5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.6 dB averaged over BWConfig Noc4 ±1.0 dB averaged over BWConfig Ês4 / Noc4 ±0.6 dB averaged over BWConfig Noc5 ±1.0 dB averaged over BWConfig Ês5 / Noc5 ±0.6 dB averaged over BWConfig Ês6 / Noc5 ±0.6 dB averaged over BWConfig Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN Noc5 is the AWGN on frequency 5 Ês5 / Noc5 is the ratio of cell 5 signal / AWGN Ês6 / Noc5 is the ratio of cell 6 signal / AWGN Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4, Ês5 / Noc5 uncertainty and Ês6 / Noc5 uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.16.72 5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX | Same as 8.16.71 | Same as 8.16.71 |
| 8.16.73 5DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX | Same as 8.16.71 | Same as 8.16.71 |
| 8.16.74 5DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX | Same as 8.16.71 | Same as 8.16.71 |
| 8.16.75 5DL PCell in FDD CA activation and deactivation of known SCell in non-DRX | Same as 8.16.67 | Same as 8.16.67 |
| 8.16.76 5DL PCell in TDD CA activation and deactivation of known SCell in non-DRX | Same as 8.16.67 | Same as 8.16.67 |
| 8.16.77 5DL FDD CA activation and deactivation of known SCell in non-DRX | Same as 8.16.67 | Same as 8.16.67 |
| 8.16.78 E-UTRAN TDD 5DL CA activation and deactivation of known SCell in non-DRX | Same as 8.16.67 | Same as 8.16.67 |
| 8.16.83 3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes | Same as 8.16.27 | Same as 8.16.27 |
| 8.16.84 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes | Same as 8.16.31 | Same as 8.16.31 |
| 8.16.85 3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes | Same as 8.16.35 | Same as 8.16.35 |
| 8.16.86 3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes | Same as 8.16.39 | Same as 8.16.39 |
| 8.16.87 4 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes | Same as 8.16.51 | Same as 8.16.51 |
| 8.16.88 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes | Same as 8.16.55 | Same as 8.16.55 |
| 8.16.91 5 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes | Same as 8.16.71 | Same as 8.16.71 |
| 8.16.92 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes | Same as 8.16.71 | Same as 8.16.71 |
| 8.16.89 4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes | Same as 8.16.57 | Same as 8.16.57 |
| 8.16.90 4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes | Same as 8.16.61 | Same as 8.16.61 |
| 8.16.93 5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes | Same as 8.16.75 | Same as 8.16.75 |
| 8.16.94 5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes | Same as 8.16.67 | Same as 8.16.67 |
| 8.16.95 6DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.6 dB averaged over BWConfig Noc4 ±1.0 dB averaged over BWConfig Ês4 / Noc4 ±0.6 dB averaged over BWConfig Noc5 ±1.0 dB averaged over BWConfig Ês5 / Noc5 ±0.6 dB averaged over BWConfig Noc6 ±1.0 dB averaged over BWConfig Ês6 / Noc6 ±0.6 dB averaged over BWConfig Ês7 / Noc6 ±0.6 dB averaged over BWConfig Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN Noc5 is the AWGN on frequency 5 Ês5 / Noc5 is the ratio of cell 5 signal / AWGN Noc6 is the AWGN on frequency 6 Ês6 / Noc6 is the ratio of cell 6 signal / AWGN Ês7 / Noc6 is the ratio of cell 7 signal / AWGN Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4, Ês5 / Noc5, Ês6 / Noc6 uncertainty and Ês7 / Noc6 uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.16.96 6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes | Same as 8.16.95 | Same as 8.16.95 |
| 8.16.97 6 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.3 dB averaged over BWConfig Noc4 ±1.0 dB averaged over BWConfig Ês4 / Noc4 ±0.3 dB averaged over BWConfig Noc5 ±1.0 dB averaged over BWConfig Ês5 / Noc5 ±0.3 dB averaged over BWConfig¨ Noc6 ±1.0 dB averaged over BWConfig Ês6 / Noc6 ±0.3 dB averaged over BWConfig Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN Noc5 is the AWGN on frequency 5 Ês5 / Noc5 is the ratio of cell 5 signal / AWGN Noc6 is the AWGN on frequency 6 Ês6 / Noc6 is the ratio of cell 6 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.16.98 6 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes | Same as 8.16.97 | Same as 8.16.97 |
| 8.16.99 7DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.6 dB averaged over BWConfig Noc4 ±1.0 dB averaged over BWConfig Ês4 / Noc4 ±0.6 dB averaged over BWConfig Noc5 ±1.0 dB averaged over BWConfig Ês5 / Noc5 ±0.6 dB averaged over BWConfig Noc6 ±1.0 dB averaged over BWConfig Ês6 / Noc6 ±0.6 dB averaged over BWConfig Noc7 ±1.0 dB averaged over BWConfig Ês7 / Noc7 ±0.6 dB averaged over BWConfig Ês8 / Noc7 ±0.6 dB averaged over BWConfig Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN Noc5 is the AWGN on frequency 5 Ês5 / Noc5 is the ratio of cell 5 signal / AWGN Noc6 is the AWGN on frequency 6 Ês6 / Noc6 is the ratio of cell 6 signal / AWGN Noc7 is the AWGN on frequency 7 Ês7 / Noc7 is the ratio of cell 7 signal / AWGN Ês8 / Noc7 is the ratio of cell 8 signal / AWGN Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4, Ês5 / Noc5, Ês6 / Noc6, Ês7 / Noc7 uncertainty and Ês8 / Noc7 uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.16.100 7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes | Same as 8.16.99 | Same as 8.16.99 |
| 8.16.101 7 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.3 dB averaged over BWConfig Noc4 ±1.0 dB averaged over BWConfig Ês4 / Noc4 ±0.3 dB averaged over BWConfig Noc5 ±1.0 dB averaged over BWConfig Ês5 / Noc5 ±0.3 dB averaged over BWConfig¨ Noc6 ±1.0 dB averaged over BWConfig Ês6 / Noc6 ±0.3 dB averaged over BWConfig Noc7 ±1.0 dB averaged over BWConfig Ês7 / Noc7 ±0.3 dB averaged over BWConfig Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN Noc5 is the AWGN on frequency 5 Ês5 / Noc5 is the ratio of cell 5 signal / AWGN Noc6 is the AWGN on frequency 6 Ês6 / Noc6 is the ratio of cell 6 signal / AWGN Noc7 is the AWGN on frequency 6 Ês7 / Noc7 is the ratio of cell 6 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.16.102 7 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes | Same as 8.16.101 | Same as 8.16.101 |
| 8.20.1 UE Measurement Procedures / RAT Measurements in CA mode / E-UTRAN FDD-FDD inter frequency event triggered reporting under fading propagation conditions in asynchronous cells | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.6 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 3 frequency Ês2 / Noc2 is the ratio of cell 3 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Ês1 / Noc1 uncertainty, Ês2 / Noc2 uncertainty, Ês3 / Noc3 uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.20.2 UE Measurement Procedures / RAT Measurements in CA mode / E-UTRAN TDD-TDD inter frequency event triggered reporting under fading propagation conditions in asynchronous cells | Same as 8.20.1 | Same as 8.20.1 |
| 8.20.2A UE Measurement Procedures / RAT Measurements in CA mode / E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth | Same as 8.20.1 | Same as 8.20.1 |
| 8.20.2B UE Measurement Procedures / RAT Measurements in CA mode / E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth | Same as 8.20.1 | Same as 8.20.1 |
| 8.20.3 E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions | E-UTRA cells Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.6 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior/Ioc ±0.6 dB CPICH Ec/Ior ±0.1 dB | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Ioc is the AWGN on Cell 2 (UTRA) frequency Ior/Ioc is the ratio of Cell 2 signal/AWGN CPICH Ec/Ior is the fraction of Cell 2 power assigned to the CPICH physical channel Ês1 / Noc1 uncertainty, Ês3 / Noc3 uncertainty or Ior/Ioc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.20.4 E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions | E-UTRA cells Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.6 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior/Ioc ±0.6 dB PCCPCH Ec/Ior ±0.1 dB DwPCH_Ec/Ior ±0.1 dB | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Ioc is the AWGN on cell 2 (UTRA) frequency Ior / Ioc is the ratio of cell 2 signal / AWGN PCCPCH Ec / Ior is the fraction of cell 2 power assigned to the PCCPCH Physical channel DwPCH_Ec/Ior is the fraction of cell 2 power assigned to the DwPCH channel Ês1 / Noc1 uncertainty, Ês3 / Noc3 uncertainty or Ior/Ioc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc or Ior/Ioc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.20.4A E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions for 20 MHz + 20 MHz bandwidth | Same as 8.20.4 | Same as 8.20.4 |
| 8.20.4B E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions for 20 MHz + 10 MHz bandwidth | Same as 8.20.4 | Same as 8.20.4 |
| 8.22.1 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal | Same as 8.1.1 | Same as 8.1.1 |
| 8.22.2 E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal | Same as 8.1.1 | Same as 8.1.1 |
| 8.22.3 E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal | Same as 8.3.1 | Same as 8.3.1 |
| 8.22.4 E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal | Same as 8.3.1 | Same as 8.3.1 |
| 8.22.5 E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal | Noc ±1.0 dB averaged over BWConfig CRS Ês1 / Noc ±0.6 dB averaged over BWConfig CRS Ês2 / Noc ±0.6 dB averaged over BWConfig CSI-RS Ês1 / Noc ±0.6 dB averaged over BWConfig CSI-RS Ês2 / Noc ±0.6 dB averaged over BWConfig | NOTE: CRS Ês1 / Noc is the ratio of cell 1 CRS signal / AWGN CRS Ês2 / Noc is the ratio of cell 2 CRS signal / AWGN CSI-RS Ês1 / Noc is the ratio of cell 1 CSI-RS signal / AWGN CSI-RS Ês2 / Noc is the ratio of cell 2 CSI-RS signal / AWGN Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.22.6 E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal | Same as 8.22.5 | Same as 8.22.5 |
| 8.22.7 E-UTRAN FDD-FDD inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal | Noc1 ±1.0 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig CSI-RS Ês1 / Noc1 ±0.6 dB averaged over BWConfig CSI-RS Ês2 / Noc2 ±0.6 dB averaged over BWConfig | NOTE: Ês1 / Noc1 is the ratio of cell 1 CRS signal / AWGN Ês2 / Noc2 is the ratio of cell 2 CRS signal / AWGN CSI-RS Ês1 / Noc1 is the ratio of cell 1 CSI-RS signal / AWGN CSI-RS Ês2 / Noc2 is the ratio of cell 2 CSI-RS signal / AWGN Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.22.8 E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal | Same as 8.22.7 | Same as 8.22.7 |
| 8.22.9 E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal | Same as 8.16.1 | Same as 8.16.1 |
| 8.22.10 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal | Same as 8.16.1 | Same as 8.16.1 |
| 8.22.11 E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal | Noc1 ±1.0 dB averaged over BWConfig CRS Ês1 / Noc1 ±0.6 dB averaged over BWConfig CSI-RS Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig CRS Ês2 / Noc2 ±0.6 dB averaged over BWConfig CRS Ês3 / Noc2 ±0.6 dB averaged over BWConfig CSI-RS Ês2 / Noc2 ±0.6 dB averaged over BWConfig CSI-RS Ês3 / Noc2 ±0.6 dB averaged over BWConfig Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 CRS Ês1 / Noc1 is the ratio of cell 1 signal / AWGN CSI-RS Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 CRS Ês2 / Noc2 is the ratio of cell 2 CRS signal / AWGN CRS Ês3 / Noc2 is the ratio of cell 3 CRS signal / AWGN CSI-RS Ês2 / Noc2 is the ratio of cell 2 CSI-RS signal / AWGN CSI-RS Ês3 / Noc2 is the ratio of cell 3 CSI-RS signal / AWGN Ês1 / Noc1, Ês2 / Noc2 uncertainty and Ês3 / Noc2 uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.22.12 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal | Same as 8.22.11 | Same as 8.22.12 |
| 8.23.1 E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês1 / Noc1 uncertainty, Ês2 / Noc2 uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.23.2 E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC | Same as 8.23.1 | Same as 8.23.1 |
| 8.23.3 E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC | Same as 8.23.1 | Same as 8.23.1 |
| 8.23.4 E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.6 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 3 frequency Ês2 / Noc2 is the ratio of cell 3 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Ês1 / Noc1 uncertainty, Ês2 / Noc2 uncertainty, Ês3 / Noc3 uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.23.5 E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC | Same as 8.23.4 | Same as 8.23.4 |
| 8.23.6 E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC | Same as 8.23.4 | Same as 8.23.4 |
| 8.26.1 E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.5dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Time alignment error cell 2 rel. to cell 1: Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.26.2 E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX | Same as 8.26.1 | Same as 8.26.1 |
| 8.26.3 E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.5dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Ês3 / Noc2 ±0.3 dB averaged over BWConfig Time alignment error cell 2 rel. to cell 1: Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.26.3A E-UTRAN FDD-FS3 3-DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.5dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Ês3 / Noc2 ±0.3 dB averaged over BWConfig Ês4 / Noc2 ±0.3 dB averaged over BWConfig Time alignment error cell 2 rel. to cell 1: Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN Ês4 / Noc2 is the ratio of cell 4 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.26.4 E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX | Same as 8.26.3 | Same as 8.26.3 |
| 8.26.5 E-UTRAN FDD-TDD intra-frequency event triggered reporting in non-DRX for CRS based discovery signal under Operation with Frame Structure 3 | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.5dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig Ês3 / Noc2 ±0.6 dB averaged over BWConfig Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.26.6 E-UTRAN TDD-TDD intra-frequency event triggered reporting in non-DRX for CRS based discovery signal under Operation with Frame Structure 3 | Same as 8.26.5 | Same as 8.26.5 |
| 8.26.7 E-UTRAN FDD-FS3 intra-frequency event triggered reporting in DRX for CRS based discovery signal | Same as 8.26.5 | Same as 8.26.5 |
| 8.26.8 E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal | Same as 8.26.5 | Same as 8.26.5 |
| 8.26.9 E-UTRAN FDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.5dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig Time alignment error cell 2 rel. to cell 1: Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês1 / Noc1, Ês2 / Noc2 uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.26.10 E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells | Same as 8.26.9 | Same as 8.26.9 |
| 9.1.1.1 FDD Intra Frequency Absolute RSRP Accuracy | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc and Ês2 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc and Ês2 / Noc each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.1.1.1_1 FDD Intra Frequency Absolute RSRP Accuracy (Rel 12 and forward) | Same as 9.1.1.1 | Same as 9.1.1.1 |
| 9.1.1.1_2 FDD Intra Frequency Absolute RSRP Accuracy for UE category 1bis | Same as 9.1.1.1 | Same as 9.1.1.1 |
| 9.1.1.1_3 FDD Intra Frequency Absolute RSRP Accuracy for CA Idle Mode Measurements | Same as 9.1.1.1 | Same as 9.1.1.1 |
| 9.1.1.2 FDD Intra Frequency Relative Accuracy of RSRP | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc and Ês2 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc and Ês2 / Noc each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.1.1.2_2 FDD Intra Frequency Relative Accuracy of RSRP for UE category 1bis | Same as 9.1.1.2 | Same as 9.1.1.2 |
| 9.1.2.1 TDD Intra Frequency Absolute RSRP Accuracy | Same as 9.1.1.1 | Same as 9.1.1.1 |
| 9.1.2.1_1 TDD Intra Frequency Absolute RSRP Accuracy (Rel 12 and forward) | Same as 9.1.1.1 | Same as 9.1.1.1 |
| 9.1.2.1_2 TDD Intra Frequency Absolute RSRP Accuracy for UE category 1bis | Same as 9.1.1.1 | Same as 9.1.1.1 |
| 9.1.2.2 TDD Intra Frequency Relative RSRP Accuracy | Same as 9.1.1.2 | Same as 9.1.1.2 |
| 9.1.2.2_2 TDD Intra Frequency Relative Accuracy of RSRP for UE category 1bis | Same as 9.1.1.2 | Same as 9.1.1.2 |
| 9.1.3.1 FDD Inter Frequency Absolute RSRP Accuracy | Noc1 and Noc2 each ±1.0 dB averaged over BWConfig Noc1 and Noc2 each ±1.3 dB for PRBs #22-27 Ês1 / Noc1 and Ês2 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 and Ês2 / Noc2 each ±0.8 dB for PRBs #22-27 | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 9.1.3.1_1 FDD Inter Frequency Absolute RSRP Accuracy (Rel 12 and forward) | Same as 9.1.3.1 | Same as 9.1.3.1 |
| 9.1.3.1_2 FDD - FDD Inter Frequency Absolute RSRP Accuracy for UE category 1bis | Same as 9.1.3.1 | Same as 9.1.3.1 |
| 9.1.3.1_3 FDD-FDD Inter Frequency Absolute RSRP Accuracy for CA Idle Mode Measurements for Overlapping Carrier | Same as 9.1.3.1 | Same as 9.1.3.1 |
| 9.1.3.1_4 FDD-FDD Inter Frequency Absolute RSRP Accuracy for CA Idle Mode Measurements for Non-Overlapping Carrier | Same as 9.1.3.1 | Same as 9.1.3.1 |
| 9.1.3.2 FDD Inter Frequency Relative RSRP Accuracy | Same as 9.1.3.1 | Same as 9.1.3.1 |
| 9.1.3.2_1 FDD Inter Frequency Relative RSRP Accuracy (Rel 12 and forward) | Same as 9.1.3.1 | Same as 9.1.3.1 |
| 9.1.3.2_2 FDD - FDD Inter Frequency Relative Accuracy of RSRP for UE category 1bis | Same as 9.1.3.1 | Same as 9.1.3.1 |
| 9.1.4.1 TDD Inter Frequency Absolute RSRP Accuracy | Same as 9.1.3.1 | Same as 9.1.3.1 |
| 9.1.4.1_1 TDD Inter Frequency Absolute RSRP Accuracy (Rel 12 and forward) | Same as 9.1.3.1 | Same as 9.1.3.1 |
| 9.1.4.1_2 TDD - TDD Inter Frequency Absolute RSRP Accuracy for UE category 1bis | Same as 9.1.3.1 | Same as 9.1.3.1 |
| 9.1.4.2 TDD Inter Frequency Relative RSRP Accuracy | Same as 9.1.3.1 | Same as 9.1.3.1 |
| 9.1.4.2_1 TDD Inter Frequency Relative RSRP Accuracy (Rel 12 and forward) | Same as 9.1.3.1 | Same as 9.1.3.1 |
| 9.1.4.2_2 TDD - TDD Inter Frequency Relative Accuracy of RSRP for UE category 1bis | Same as 9.1.3.1 | Same as 9.1.3.1 |
| 9.1.5.1 FDD - TDD Inter Frequency Absolute RSRP Accuracy | Noc1 and Noc2 each ±1.0 dB averaged over BWConfig Noc1 and Noc2 each ±1.3 dB for PRBs #22-27 Ês1 / Noc1 and Ês2 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 and Ês2 / Noc2 each ±0.8 dB for PRBs #22-27 | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 9.1.5.1_1 FDD - TDD Inter Frequency Absolute RSRP Accuracy (Rel 12 and forward) | Same as 9.1.5.1 | Same as 9.1.5.1 |
| 9.1.5.1_2 FDD - TDD Inter Frequency Absolute RSRP Accuracy for UE category 1bis | Same as 9.1.5.1 | Same as 9.1.5.1 |
| 9.1.5.2 FDD - TDD Inter Frequency Relative Accuracy of RSRP | Noc1 and Noc2 each ±1.0 dB averaged over BWConfig Noc1 and Noc2 each ±1.3 dB for PRBs #22-27 Ês1 / Noc1 and Ês2 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 and Ês2 / Noc2 each ±0.8 dB for PRBs #22-27 | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 9.1.5.2_1 FDD - TDD Inter Frequency Relative Accuracy of RSRP (Rel 12 and forward) | Same as 9.1.5.2 | Same as 9.1.5.2 |
| 9.1.5.2_2 FDD - TDD Inter Frequency Relative Accuracy of RSRP for UE category 1bis | Same as 9.1.5.2 | Same as 9.1.5.2 |
| 9.1.6.1 FDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #22-27 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.1.6.1_1 Measurement Performance Requirements / FDD Absolute RSRP Accuracy E-UTRA for Carrier Aggregation (Rel 12 and forward) | Same as 9.1.6.1 | Same as 9.1.6.1 |
| 9.1.6.2 FDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation | Same as 9.1.6.1 |  |
| 9.1.6.2_1 FDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation (Rel 12 and forward) | Same as 9.1.6.1 | Same as 9.1.6.1 |
| 9.1.7.1 TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation | Same as 9.1.6.1 |  |
| 9.1.7.1_1 TDD Absolute RSRP Accuracy E-UTRA for Carrier Aggregation (Rel 12 and forward) | Same as 9.1.6.1 |  |
| 9.1.7.2 TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation | Same as 9.1.6.1 |  |
| 9.1.7.2_1 TDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation (Rel 12 and forward) | Same as 9.1.6.2_1 | Same as 9.1.6.2_1 |
| 9.1.8.1 FDD Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with Non-MBSFN ABS | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc and Ês2 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc and Ês2 / Noc each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.1.8.1_1 FDD Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) (Rel-12 and forward) | Same as 9.1.8.1 | Same as 9.1.8.1 |
| 9.1.8.2 FDD Relative RSRP Accuracy under Time Domain Measurement Resource Restriction with Non-MBSFN ABS | Same as 9.1.8.1 |  |
| 9.1.9.1 TDD Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with Non-MBSFN ABS | Same as 9.1.8.1 |  |
| 9.1.9.1_1 TDD Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) (Rel 12 and forward) | Same as 9.1.8.1 | Same as 9.1.8.1 |
| 9.1.9.2 TDD Relative RSRP Accuracy under Time Domain Measurement Resource Restriction with Non-MBSFN ABS | Same as 9.1.8.1 |  |
| 9.1.10.1 FDD Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with MBSFN ABS | Same as 9.1.8.1 |  |
| 9.1.10.1_1 FDD Absolute RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Rel-12 and forward) | Same as 9.1.8.1 |  |
| 9.1.10.2 FDD Relative RSRP Accuracy under Time Domain Measurement Resource Restriction with MBSFN ABS | Same as 9.1.8.1 |  |
| 9.1.11.1 TDD Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with MBSFN ABS | Same as 9.1.8.1 |  |
| 9.1.11.1_1 TDD Absolute RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Rel 12 and forward) | Same as 9.1.8.1 |  |
| 9.1.11.2 TDD Relative RSRP Accuracy under Time Domain Measurement Resource Restriction with MBSFN ABS | Same as 9.1.8.1 |  |
| 9.1.12.1 FDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #47-52 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #47-52 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #47-52 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.1.12.1_1 Measurement Performance Requirements / FDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz (Rel 12 and forward) | Same as 9.1.12.1 | Same as 9.1.12.1 |
| 9.1.12.2 FDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz | Same as 9.1.12.1 |  |
| 9.1.12.2_1 FDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz (Rel 12 and forward) | Same as 9.1.12.1 | Same as 9.1.12.1 |
| 9.1.13.1 TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz | Same as 9.1.12.1 |  |
| 9.1.13.1_1 TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz (Rel 12 and forward) | Same as 9.1.12.1 |  |
| 9.1.13.2 TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz | Same as 9.1.12.1 |  |
| 9.1.13.2_1 TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz (Rel 12 and forward) | Same as 9.1.12.2_1 | Same as 9.1.12.2_1 |
| 9.1.14.1 FDD Intra Frequency Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc, Ês2 / Noc and Ês3 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc, Ês2 / Noc and Ês3 / Noc each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês3 / Noc is the ratio of cell 2 signal / AWGN |
| 9.1.14.1_1 FDD Intra Frequency Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) (Rel 12 and forward) | Same as 9.1.14.1 | Same as 9.1.14.1 |
| 9.1.14.2 FDD Intra Frequency Relative RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc, Ês2 / Noc and Ês3 / Noc each ±0.3 dB averaged over BWConfigÊs1 / Noc, Ês2 / Noc and Ês3 / Noc each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês3 / Noc is the ratio of cell 3 signal / AWGN |
| 9.1.15.1 TDD Intra Frequency Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Same as 9.1.14.1 | Same as 9.1.14.1 |
| 9.1.15.2 TDD Intra Frequency Relative RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Same as 9.1.14.2 |  |
| 9.1.16.1 FDD Intra Frequency Absolute RSRP Accuracy for 5MHz Bandwidth | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #10-15 Ês1 / Noc and Ês2 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc and Ês2 / Noc each ±0.8 dB for PRBs #10-15 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.1.16.1_1 FDD Intra Frequency Absolute RSRP Accuracy for 5MHz Bandwidth (Rel 12 and forward) | Same as 9.1.16.1 | Same as 9.1.16.1 |
| 9.1.16.2 FDD Intra Frequency Relative Accuracy of RSRP for 5MHz Bandwidth | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #10-15 Ês1 / Noc and Ês2 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc and Ês2 / Noc each ±0.8 dB for PRBs #10-15 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.1.17.1 FDD Inter Frequency Absolute RSRP Accuracy for 5MHz Bandwidth | Noc1 and Noc2 each ±1.0 dB averaged over BWConfig Noc1 and Noc2 each ±1.3 dB for PRBs #10-15 Ês1 / Noc1 and Ês2 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 and Ês2 / Noc2 each ±0.8 dB for PRBs #10-15 | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 9.1.17.2 FDD - FDD Inter Frequency Relative Accuracy of RSRP for 5MHz Bandwidth | Noc1 and Noc2 each ±1.0 dB averaged over BWConfig Noc1 and Noc2 each ±1.3 dB for PRBs #10-15 Ês1 / Noc1 and Ês2 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 and Ês2 / Noc2 each ±0.8 dB for PRBs #10-15 | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 9.1.18.1 FDD Absolute RSRP Accuracy for E-UTRA for Carrier Aggregation for 10MHz + 5MHz | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #10-15 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 ±0.8 dB for PRBs #22-27 Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #10-15 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.1.18.2 FDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation for 10MHz + 5MHz | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #10-15 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 ±0.8 dB for PRBs #22-27 Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #10-15 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.1.18.2_1 FDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation for 10MHz + 5MHz (Rel-12 and forward) | Same as 9.1.18.2 | Same as 9.1.18.2 |
| 9.1.19.1 TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 10MHz + 5MHz | Same as 9.1.18.1 | Same as 9.1.18.1 |
| 9.1.19.2 TDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation for 10MHz + 5MHz | Same as 9.1.18.2 | Same as 9.1.18.2 |
| 9.1.19.2_1 TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 10MHz + 5MHz (Rel-12 and forward) | Same as 9.1.18.2 | Same as 9.1.18.2 |
| 9.1.20.1 FDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 5MHz + 5MHz bandwidth | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #10-15 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #10-15 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #10-15 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.1.20.2 FDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 5MHz + 5MHz bandwidth | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #10-15 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #10-15 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #10-15 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.1.21.1 TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 5MHz + 5MHz bandwidth | Same as 9.1.20.1 | Same as 9.1.20.1 |
| 9.1.21.2 TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 5MHz + 5MHz bandwidth | Same as 9.1.20.2 | Same as 9.1.20.2 |
| 9.1.22 FDD-TDD RSRP Accuracy E-UTRA for Carrier Aggregation with PCell in FDD | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB over UE Measurement bandwidth Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB over UE Measurement bandwidth Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc2 each ±0.8 dB over UE Measurement bandwidth Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | Notes: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN UE Measurement bandwidth: 5MHz Ch BW: PRBs 10-15 10MHz Ch BW: PRBs 22-27 20MHz Ch BW: PRBs 47-52 TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.1.23 FDD-TDD RSRP Accuracy E-UTRA for Carrier Aggregation with PCell in TDD | Same as 9.1.22 | Same as 9.1.22 |
| 9.1.24.1 TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz + 10MHz | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #47-52 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 ±0.8 dB for PRBs #47-52 Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #22-27 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.1.24.1_1 TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz + 10MHz (Rel-12 and forward) | Same as 9.1.24.1 | Same as 9.1.24.1 |
| 9.1.24.2 TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz + 10MHz | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #47-52 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 ±0.8 dB for PRBs #47-52 Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #22-27 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.1.24.2_1 TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz + 10MHz (Rel-12 and forward) | Same as 9.1.24.2 | Same as 9.1.24.2 |
| 9.1.25 FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal | Same as 9.1.1.1 | Same as 9.1.1.1 |
| 9.1.26 TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal | Same as 9.1.1.1 | Same as 9.1.1.1 |
| 9.1.27 FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1, Ês2 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2 each ±0.8 dB for PRBs #22-27 | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 9.1.28 TDD—TDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal | Same as 9.1.27 | Same as 9.1.27 |
| 9.1.29 FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 CRS Ês1 / Noc and CRS Ês2 / Noc each ±0.3 dB averaged over BWConfig CSI-RS Ês1 / Noc and CSI-RS Ês2 / Noc each ±0.3 dB averaged over BWConfig CRS Ês1 / Noc and CRS Ês2 / Noc each ±0.8 dB for PRBs #22-27 CSI-RS Ês1 / Noc and CSI-RS Ês2 / Noc each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.1.30 TDD intra-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal | Same as 9.1.29 | Same as 9.1.29 |
| 9.1.31 FDD-FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1, Ês2 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2 each ±0.8 dB for PRBs #22-27 | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 9.1.32 TDD-TDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal | Same as 9.1.32 | Same as 9.1.32 |
| 9.1.33 FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #22-27 | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN |
| 9.1.34 TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal | Same as 9.1.33 | Same as 9.1.33 |
| 9.1.35 FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 CSI-RS Ês1 / Noc1, CSI-RS Ês2 / Noc2, and CSI-RS Ês3 / Noc3 each ±0.3 dB averaged over BWConfig CSI-RS Ês1 / Noc1, CSI-RS Ês2 / Noc2, and CSI-RS Ês3 / Noc3 each ±0.8 dB for PRBs #22-27 | NOTE: Noc1 is the AWGN on frequency 1 CSI-RS Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 CSI-RS Ês2 / Noc2 is the ratio of cell 2 signal / AWGN CSI-RS Ês3 / Noc2 is the ratio of cell 3 signal / AWGN |
| 9.1.36 TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal | Same as 9.1.35 | Same as 9.1.35 |
| 9.1.37 3DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB over UE Measurement bandwidth Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB over UE Measurement bandwidth Noc3 ±1.0 dB averaged over BWConfig Noc3 ±1.3 dB over UE Measurement bandwidth Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc2, Ês4 / Noc3, Ês5 / Noc3 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc2, Ês4 / Noc3, Ês5 / Noc3 each ±0.8 dB over UE Measurement bandwidth Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | Notes: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN Noc3 is the AWGN on frequency 3 Ês4 / Noc3 is the ratio of cell 4 signal / AWGN Ês5 / Noc3 is the ratio of cell 5 signal / AWGN UE Measurement bandwidth: 5MHz Ch BW: PRBs 10-15 10MHz Ch BW: PRBs 22-27 20MHz Ch BW: PRBs 47-52 TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.1.38 3DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation | Same as 9.1.37 | Same as 9.1.37 |
| 9.1.39 3DL FDD RSRP for E-UTRAN in Carrier Aggregation | Same as 9.1.37 | Same as 9.1.37 |
| 9.1.39_1 3DL FDD RSRP for E-UTRAN in Carrier Aggregation (Rel-12 and forward) | Same as 9.1.37 | Same as 9.1.37 |
| 9.1.40 3DL TDD RSRP for E-UTRAN in Carrier Aggregation | Same as 9.1.37 | Same as 9.1.37 |
| 9.1.40_1 3DL TDD RSRP for E-UTRAN in Carrier Aggregation (Rel-12 and forward) | Same as 9.1.37 | Same as 9.1.37 |
| 9.1.41.1 FD-FDD Intra Frequency Absolute RSRP Accuracy for UE category 0 | Same as 9.1.1.1 | Same as 9.1.1.1 |
| 9.1.41.2 FD-FDD Intra Frequency Relative RSRP Accuracy for UE category 0 | Same as 9.1.1.2 | Same as 9.1.1.2 |
| 9.1.42.1 HD-FDD Intra Frequency Absolute RSRP Accuracy for UE category 0 | Same as 9.1.1.1 | Same as 9.1.1.1 |
| 9.1.42.2 HD-FDD Intra Frequency Relative RSRP Accuracy for UE category 0 | Same as 9.1.1.2 | Same as 9.1.1.2 |
| 9.1.43.1 TDD Intra Frequency Absolute RSRP Accuracy for UE category 0 | Same as 9.1.1.1 | Same as 9.1.1.1 |
| 9.1.43.2 TDD Intra Frequency Relative RSRP Accuracy for UE category 0 | Same as 9.1.1.2 | Same as 9.1.1.2 |
| 9.1.44 4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB over UE Measurement bandwidth Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB over UE Measurement bandwidth Noc3 ±1.0 dB averaged over BWConfig Noc3 ±1.3 dB over UE Measurement bandwidth Noc4 ±1.0 dB averaged over BWConfig Noc4 ±1.3 dB over UE Measurement bandwidth Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc2, Ês4 / Noc3, Ês5 / Noc3, Ês6 / Noc4, Ês7 / Noc4 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc2, Ês4 / Noc3, Ês5 / Noc3, Ês6 / Noc4, Ês7 / Noc4 each ±0.8 dB over UE Measurement bandwidth Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | Notes: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN Noc3 is the AWGN on frequency 3 Ês4 / Noc3 is the ratio of cell 4 signal / AWGN Ês5 / Noc3 is the ratio of cell 5 signal / AWGN Noc4 is the AWGN on frequency 4 Ês6 / Noc4 is the ratio of cell 6 signal / AWGN Ês7 / Noc4 is the ratio of cell 7 signal / AWGN UE Measurement bandwidth: 5MHz Ch BW: PRBs 10-15 10MHz Ch BW: PRBs 22-27 20MHz Ch BW: PRBs 47-52 TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.1.45 4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation | Same as 9.1.44 | Same as 9.1.44 |
| 9.1.46 4DL FDD RSRP for E-UTRAN in Carrier Aggregation | Same as 9.1.44 | Same as 9.1.44 |
| 9.1.47 4DL TDD RSRP for E-UTRAN in Carrier Aggregation | Same as 9.1.44 | Same as 9.1.44 |
| 9.1.48 5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB over UE Measurement bandwidth Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB over UE Measurement bandwidth Noc3 ±1.0 dB averaged over BWConfig Noc3 ±1.3 dB over UE Measurement bandwidth Noc4 ±1.0 dB averaged over BWConfig Noc4 ±1.3 dB over UE Measurement bandwidth Noc5 ±1.0 dB averaged over BWConfig Noc5 ±1.3 dB over UE Measurement bandwidth Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc2, Ês4 / Noc3, Ês5 / Noc3, Ês6 / Noc4, Ês7 / Noc4 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc2, Ês4 / Noc3, Ês5 / Noc3, Ês6 / Noc4, Ês7 / Noc4, Ês8 / Noc5, Ês8 / Noc5 each ±0.8 dB over UE Measurement bandwidth Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | Notes: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN Noc3 is the AWGN on frequency 3 Ês4 / Noc3 is the ratio of cell 4 signal / AWGN Ês5 / Noc3 is the ratio of cell 5 signal / AWGN Noc4 is the AWGN on frequency 4 Ês6 / Noc4 is the ratio of cell 6 signal / AWGN Ês7 / Noc4 is the ratio of cell 7 signal / AWGN Noc5 is the AWGN on frequency 5 Ês8 / Noc5 is the ratio of cell 8 signal / AWGN Ês9 / Noc5 is the ratio of cell 9 signal / AWGN UE Measurement bandwidth: 5MHz Ch BW: PRBs 10-15 10MHz Ch BW: PRBs 22-27 20MHz Ch BW: PRBs 47-52 TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.1.49 5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation | Same as 9.1.48 | Same as 9.1.48 |
| 9.1.50 5 DL FDD RSRP for E-UTRAN in Carrier Aggregation | Same as 9.1.48 | Same as 9.1.48 |
| 9.1.51 5 DL TDD RSRP for E-UTRAN in Carrier Aggregation | Same as 9.1.48 | Same as 9.1.48 |
| 9.1.52 FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc and Ês2 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc and Ês2 / Noc each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.1.53 HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | Same as 9.1.52 | Same as 9.1.52 |
| 9.1.54 TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | Same as 9.1.52 | Same as 9.1.52 |
| 9.1.55 FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell9.1.55 FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #47-52 Noc2 ±1.5 dB averaged over BWConfig Noc2 ±1.8 dB for PRBs #47-52 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #47-52 | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGNNOTE: |
| 9.1.56 FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell9.1.56 FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell | Same as 9.1.55 | Same as 9.1.55 |
| 9.1.57 FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc, Ês2 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc, Ês2 / Noc each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.1.58 HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB | Same as 9.1.57 | Same as 9.1.57 |
| 9.1.59 TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB | Same as 9.1.57 | Same as 9.1.57 |
| 9.1.60 FS3 absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal with FDD PCell | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB over UE Measurement bandwidth Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB over UE Measurement bandwidth CRS Ês1 / Noc1, CRS Ês2 / Noc2, CRS Ês3 / Noc2 each ±0.3 dB averaged over BWConfig CSI-RS Ês2 / Noc2, CSI-RS Ês3 / Noc2 each ±0.3 dB averaged over BWConfig CRS Ês2 / Noc2, CRS Ês3 / Noc2 each ±0.8 dB over UE Measurement bandwidth CSI-RS Ês1 / Noc1, CSI-RS Ês2 / Noc2, CSI-RS Ês3 / Noc2 each ±0.8 dB over UE Measurement bandwidth | Notes: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN |
| 9.1.61 FS3 absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal with TDD PCell | Same as 9.1.60 | Same as 9.1.60 |
| 9.1.68 3DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes | Same as 9.1.37 | Same as 9.1.37 |
| 9.1.69 4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes | Same as 9.1.44 | Same as 9.1.44 |
| 9.1.70 5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes | Same as 9.1.48 | Same as 9.1.48 |
| 9.1.71 6 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB over UE Measurement bandwidth Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB over UE Measurement bandwidth Noc3 ±1.0 dB averaged over BWConfig Noc3 ±1.3 dB over UE Measurement bandwidth Noc4 ±1.0 dB averaged over BWConfig Noc4 ±1.3 dB over UE Measurement bandwidth Noc5 ±1.0 dB averaged over BWConfig Noc5 ±1.3 dB over UE Measurement bandwidth Noc6 ±1.0 dB averaged over BWConfig Noc6 ±1.3 dB over UE Measurement bandwidth Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc2, Ês4 / Noc3, Ês5 / Noc3, Ês6 / Noc4, Ês7 / Noc4, Ês8 / Noc5, Ês9 / Noc5, Ês10 / Noc6, Ês11 / Noc6 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc2, Ês4 / Noc3, Ês5 / Noc3, Ês6 / Noc4, Ês7 / Noc4, Ês8 / Noc5, Ês9 / Noc5, Ês10 / Noc6, Ês11 / Noc6 each ±0.8 dB over UE Measurement bandwidth Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | Notes: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN Noc3 is the AWGN on frequency 3 Ês4 / Noc3 is the ratio of cell 4 signal / AWGN Ês5 / Noc3 is the ratio of cell 5 signal / AWGN Noc4 is the AWGN on frequency 4 Ês6 / Noc4 is the ratio of cell 6 signal / AWGN Ês7 / Noc4 is the ratio of cell 7 signal / AWGN Noc5 is the AWGN on frequency 5 Ês8 / Noc5 is the ratio of cell 8 signal / AWGN Ês9 / Noc5 is the ratio of cell 9 signal / AWGN Noc6 is the AWGN on frequency 6 Ês10 / Noc6 is the ratio of cell 10 signal / AWGN Ês11 / Noc6 is the ratio of cell 11 signal / AWGN UE Measurement bandwidth: 5MHz Ch BW: PRBs 10-15 10MHz Ch BW: PRBs 22-27 20MHz Ch BW: PRBs 47-52 TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] |
| 9.1.72 7 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB over UE Measurement bandwidth Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB over UE Measurement bandwidth Noc3 ±1.0 dB averaged over BWConfig Noc3 ±1.3 dB over UE Measurement bandwidth Noc4 ±1.0 dB averaged over BWConfig Noc4 ±1.3 dB over UE Measurement bandwidth Noc5 ±1.0 dB averaged over BWConfig Noc5 ±1.3 dB over UE Measurement bandwidth Noc6 ±1.3 dB over UE Measurement bandwidth Noc6 ±1.0 dB averaged over BWConfig Noc7 ±1.3 dB over UE Measurement bandwidth Noc7 ±1.0 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc2, Ês4 / Noc3, Ês5 / Noc3, Ês6 / Noc4, Ês7 / Noc4, Ês8 / Noc5, Ês9 / Noc5, Ês10 / Noc6, Ês11 / Noc6, Ês12 / Noc7, Ês13 / Noc7 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc2, Ês4 / Noc3, Ês5 / Noc3, Ês6 / Noc4, Ês7 / Noc4, Ês8 / Noc5, Ês9 / Noc5, Ês10 / Noc6, Ês11 / Noc6, Ês12 / Noc7, Ês13 / Noc7 each ±0.8 dB over UE Measurement bandwidth Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | Notes: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN Noc3 is the AWGN on frequency 3 Ês4 / Noc3 is the ratio of cell 4 signal / AWGN Ês5 / Noc3 is the ratio of cell 5 signal / AWGN Noc4 is the AWGN on frequency 4 Ês6 / Noc4 is the ratio of cell 6 signal / AWGN Ês7 / Noc4 is the ratio of cell 7 signal / AWGN Noc5 is the AWGN on frequency 5 Ês8 / Noc5 is the ratio of cell 8 signal / AWGN Ês9 / Noc5 is the ratio of cell 9 signal / AWGN Noc6 is the AWGN on frequency 6 Ês10 / Noc6 is the ratio of cell 10 signal / AWGN Ês11 / Noc6 is the ratio of cell 11 signal / AWGN Noc7 is the AWGN on frequency 7 Ês12 / Noc7 is the ratio of cell 12 signal / AWGN Ês13 / Noc7 is the ratio of cell 13 signal / AWGN UE Measurement bandwidth: 5MHz Ch BW: PRBs 10-15 10MHz Ch BW: PRBs 22-27 20MHz Ch BW: PRBs 47-52 TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] |
| 9.2.1.1 FDD Intra Frequency Absolute RSRQ Accuracy | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-2 7 Ês1 / Noc and Ês2 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc and Ês2 / Noc each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.2.1.1_1 FDD Intra Frequency Absolute RSRQ Accuracy for UE Category 1bis | Same as 9.2.1.1 | Same as 9.2.1.1 |
| 9.2.1.1_2 FDD Intra Frequency Absolute RSRQ Accuracy for UE category 1bis | Same as 9.2.1.1 | Same as 9.2.1.1 |
| 9.2.1.1_3 FDD Intra Frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements | Same as 9.2.1.1 | Same as 9.2.1.1 |
| 9.2.2.1 TDD Intra Frequency Absolute RSRQ Accuracy | Same as 9.2.1.1 | Same as 9.2.1.1 |
| 9.2.2.1_1 TDD Intra Frequency Absolute RSRQ Accuracy for UE Category 1bis | Same as 9.2.1.1 | Same as 9.2.1.1 |
| 9.2.2.1_2 TDD Intra Frequency Absolute RSRQ Accuracy for UE category 1bis | Same as 9.2.1.1 | Same as 9.2.1.1 |
| 9.2.3.1 FDD - FDD Inter Frequency Absolute RSRQ Accuracy | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1 and Ês2 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 and Ês2 / Noc2 each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN on frequency 1 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN on frequency 2 |
| 9.2.3.1_2 FDD - FDD Inter Frequency Absolute RSRQ Accuracy for UE Category 1bis | Same as 9.2.3.1 | Same as 9.2.3.1 |
| 9.2.3.1_3 FDD - FDD Inter Frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements for Overlapping Carrier | Same as 9.2.3.1 | Same as 9.2.3.1 |
| 9.2.3.1_4 FDD - FDD Inter Frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements for Non-Overlapping Carrier | Same as 9.2.3.1 | Same as 9.2.3.1 |
| 9.2.3.2 FDD - FDD Inter Frequency Relative RSRQ Accuracy | Same as 9.2.3.1 |  |
| 9.2.3.2_2 FDD - FDD Inter Frequency Relative RSRQ Accuracy for UE Category 1bis | Same as 9.2.3.1 | Same as 9.2.3.1 |
| 9.2.4.1 TDD - TDD Inter Frequency Absolute RSRQ Accuracy | Same as 9.2.3.1 |  |
| 9.2.4.1_1 TDD - TDD Inter Frequency Absolute RSRQ Accuracy (Rel-12 and forward) | Same as 9.2.3.1 |  |
| 9.2.4.1_2 TDD - TDD Inter Frequency Absolute RSRQ Accuracy for UE Category 1bis | Same as 9.2.3.1 | Same as 9.2.3.1 |
| 9.2.4.2 TDD - TDD Inter Frequency Relative RSRQ Accuracy | Same as 9.2.3.1 |  |
| 9.2.4.2_1 TDD - TDD Inter Frequency Relative RSRQ Accuracy (Rel-12 and forward) | Same as 9.2.3.2 |  |
| 9.2.4.2_2 TDD - TDD Inter Frequency Relative RSRQ Accuracy for UE Category 1bis | Same as 9.2.3.1 | Same as 9.2.3.1 |
| 9.2.4A.1 FDD - TDD Inter Frequency Absolute RSRQ Accuracy | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1 and Ês2 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 and Ês2 / Noc2 each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN on frequency 1 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN on frequency 2 |
| 9.2.4A.1_2 FDD - TDD Inter Frequency Absolute RSRQ Accuracy for UE Category 1bis | Same as 9.2.4A.1 | Same as 9.2.4A.1 |
| 9.2.4A.2 FDD - TDD Inter Frequency Relative Accuracy of RSRQ | Same as 9.2.4A.1 |  |
| 9.2.4A.2_2 FDD - TDD Inter Frequency Relative Accuracy of RSRQ for UE Category 1bis | Same as 9.2.4A.1 | Same as 9.2.4A.1 |
| 9.2.5.1 FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #22-27 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.2.5.2 FDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation | Same as 9.2.5.1 |  |
| 9.2.6.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation | Same as 9.2.5.1 |  |
| 9.2.6.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation | Same as 9.2.5.1 |  |
| 9.2.7.1 FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc and Ês2 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc and Ês2 / Noc each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.2.8.1 TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS | Same as 9.2.7.1 | Same as 9.2.7.1 |
| 9.2.9.1 FDD Absolute RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc and Ês2 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc and Ês2 / Noc each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.2.10.1 TDD Absolute RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) | Same as 9.2.9.1 |  |
| 9.2.11.1 FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #47-52 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #47-52 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #47-52 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.2.11.2 FDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz | Same as 9.2.11.1 |  |
| 9.2.12.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz | Same as 9.2.11.1 |  |
| 9.2.12.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz | Same as 9.2.11.1 |  |
| 9.2.15.1 FDD RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc, Ês2 / Noc and Ês3 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc, Ês2 / Noc and Ês3 / Noc each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês3 / Noc is the ratio of cell 3 signal / AWGN |
| 9.2.16.1 TDD RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Same as 9.2.15.1 | Same as 9.2.15.1 |
| 9.2.17.1 FDD Intra Frequency Absolute RSRQ Accuracy for 5MHz Bandwidth | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #10-15 Ês1 / Noc and Ês2 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc and Ês2 / Noc each ±0.8 dB for PRBs #10-15 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.2.18.1 FDD - FDD Inter Frequency Absolute RSRQ Accuracy for 5MHz Bandwidth | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #10-15 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #10-15 Ês1 / Noc1 and Ês2 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 and Ês2 / Noc2 each ±0.8 dB for PRBs #10-15 | NOTE: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN on frequency 1 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN on frequency 2 |
| 9.2.18.2 FDD - FDD Inter Frequency Relative RSRQ Accuracy for 5MHz Bandwidth | Same as 9.2.18.1 |  |
| 9.2.19.1 FDD-FDD Inter Frequency absolute WB-RSRQ accuracy | FFS | FFS |
| 9.2.20.1 TDD-TDD Inter Frequency absolute WB-RSRQ accuracy | FFS | FFS |
| 9.2.21.1 FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #10-15 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 ±0.8 dB for PRBs #22-27 Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #10-15 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.2.21.2 FDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz | Same as 9.2.21.1 |  |
| 9.2.22.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz | Same as 9.2.21.1 |  |
| 9.2.22.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz | Same as 9.2.21.1 |  |
| 9.2.23.1 FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #10-15 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #10-15 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #10-15 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.2.23.2 FDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz | Same as 9.2.23.1 | Same as 9.2.23.1 |
| 9.2.24.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz | Same as 9.2.23.1 | Same as 9.2.23.1 |
| 9.2.24.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz | Same as 9.2.23.1 | Same as 9.2.23.1 |
| 9.2.25.1 Absolute RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1 and Ês2 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 and Ês2 / Noc2 each ±0.8 dB for PRBs #22-27 Time alignment error cell 2 rel. to cell 1: Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.2.25.2 Relative RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD | Same as 9.2.25.1 |  |
| 9.2.26.1 Absolute RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD | Same as 9.2.25.1 |  |
| 9.2.26.2 Relative RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD | Same as 9.2.25.1 |  |
| 9.2.27.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz+10MHz | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #47-52 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 ±0.8 dB for PRBs #47-52 Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #22-27 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.2.27.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz+10MHz | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #47-52 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 ±0.8 dB for PRBs #47-52 Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #22-27 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.2.28 FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc and Ês2 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc and Ês2 / Noc each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.2.29 TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal | Same as 9.2.28 | Same as 9.2.28 |
| 9.2.30 FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1 and Ês2 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 and Ês2 / Noc2 each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN on frequency 1 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN on frequency 2 |
| 9.2.31 TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal | Same as 9.2.30 | Same as 9.2.30 |
| 9.2.32 FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #22-27 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.2.33 TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal | Same as 9.2.32 | Same as 9.2.32 |
| 9.2.38 3DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB over UE Measurement bandwidth Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB over UE Measurement bandwidth Noc3 ±1.0 dB averaged over BWConfig Noc3 ±1.3 dB over UE Measurement bandwidth Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc3 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc3 each ±0.8 dB over UE Measurement bandwidth Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | Notes: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN UE Measurement bandwidth: 5MHz Ch BW: PRBs 10-15 10MHz Ch BW: PRBs 22-27 20MHz Ch BW: PRBs 47-52 TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.2.39 3DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation | Same as 9.2.38 | Same as 9.2.38 |
| 9.2.40 3DL FDD RSRQ for E-UTRA Carrier Aggregation | Same as 9.2.38 | Same as 9.2.38 |
| 9.2.41 3DL TDD RSRQ for E-UTRA Carrier Aggregation | Same as 9.2.38 | Same as 9.2.38 |
| 9.2.42.1 FD-FDD Intra Frequency Absolute RSRQ Accuracy for UE category 0 | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc and Ês2 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc and Ês2 / Noc each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.2.43.1 HD-FDD Intra Frequency Absolute RSRQ Accuracy for UE category 0 | Same as 9.2.42.1 | Same as 9.2.42.1 |
| 9.2.44.1 TDD Intra Frequency Absolute RSRQ Accuracy for UE category 0 | Same as 9.2.42.1 | Same as 9.2.42.1 |
| 9.2.45 4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB over UE Measurement bandwidth Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB over UE Measurement bandwidth Noc3 ±1.0 dB averaged over BWConfig Noc3 ±1.3 dB over UE Measurement bandwidth Noc4 ±1.0 dB averaged over BWConfig Noc4 ±1.3 dB over UE Measurement bandwidth Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4 each ±0.8 dB over UE Measurement bandwidth Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | Notes: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN UE Measurement bandwidth: 5MHz Ch BW: PRBs 10-15 10MHz Ch BW: PRBs 22-27 20MHz Ch BW: PRBs 47-52 Ts = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.2.46 4DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation | Same as 9.2.45 | Same as 9.2.45 |
| 9.2.47 5 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB over UE Measurement bandwidth Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB over UE Measurement bandwidth Noc3 ±1.0 dB averaged over BWConfig Noc3 ±1.3 dB over UE Measurement bandwidth Noc4 ±1.0 dB averaged over BWConfig Noc4 ±1.3 dB over UE Measurement bandwidth Noc5 ±1.0 dB averaged over BWConfig Noc5 ±1.3 dB over UE Measurement bandwidth Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4, Ês5 / Noc5 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4, Ês5 / Noc5 each ±0.8 dB over UE Measurement bandwidth Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | Notes: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN Noc4 is the AWGN on frequency 5 Ês5 / Noc5 is the ratio of cell 5 signal / AWGN UE Measurement bandwidth: 5MHz Ch BW: PRBs 10-15 10MHz Ch BW: PRBs 22-27 20MHz Ch BW: PRBs 47-52 Ts = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.2.48 5 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation | Same as 9.2.47 | Same as 9.2.47 |
| 9.2.51 FDD intra frequency absolute and relative RSRQ accuracies for SCell with frame structure 3 | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB averaged over UE Measurement bandwidth Noc2 ±1.5 dB averaged over BWConfig Noc2 ±1.8 dB for PRBs #47-52 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB averaged over UE Measurement bandwidth Time alignment error cell 2 rel. to cell 1: Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.2.52 TDD intra frequency absolute and relative RSRQ accuracies for SCell with frame structure 3 | Same as 9.2.51 | Same as 9.2.51 |
| 9.2.55 3DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes | Same as 9.2.38 | Same as 9.2.38 |
| 9.2.56 4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes | Same as 9.2.45 | Same as 9.2.45 |
| 9.2.57 5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes | Same as 9.2.47 | Same as 9.2.47 |
| 9.2.58 6 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB over UE Measurement bandwidth Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB over UE Measurement bandwidth Noc3 ±1.0 dB averaged over BWConfig Noc3 ±1.3 dB over UE Measurement bandwidth Noc4 ±1.0 dB averaged over BWConfig Noc4 ±1.3 dB over UE Measurement bandwidth Noc5 ±1.0 dB averaged over BWConfig Noc5 ±1.3 dB over UE Measurement bandwidth Noc6 ±1.0 dB averaged over BWConfig Noc6 ±1.3 dB over UE Measurement bandwidth Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4, Ês5 / Noc5, Ês6 / Noc6 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4, Ês5 / Noc5ㄝÊs6 / Noc6 each ±0.8 dB over UE Measurement bandwidth Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | Notes: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN Noc5 is the AWGN on frequency 5 Ês5 / Noc5 is the ratio of cell 5 signal / AWGN Noc6 is the AWGN on frequency 6 Ês6 / Noc6 is the ratio of cell 6 signal / AWGN UE Measurement bandwidth: 5MHz Ch BW: PRBs 10-15 10MHz Ch BW: PRBs 22-27 20MHz Ch BW: PRBs 47-52 Ts = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] |
| 9.2.59 7 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB over UE Measurement bandwidth Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB over UE Measurement bandwidth Noc3 ±1.0 dB averaged over BWConfig Noc3 ±1.3 dB over UE Measurement bandwidth Noc4 ±1.0 dB averaged over BWConfig Noc4 ±1.3 dB over UE Measurement bandwidth Noc5 ±1.0 dB averaged over BWConfig Noc5 ±1.3 dB over UE Measurement bandwidth Noc6 ±1.0 dB averaged over BWConfig Noc6 ±1.3 dB over UE Measurement bandwidth Noc7 ±1.0 dB averaged over BWConfig Noc7 ±1.3 dB over UE Measurement bandwidth Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4, Ês5 / Noc5, Ês6 / Noc6, Ês7 / Noc7 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4, Ês5 / Noc5, Ês6 / Noc6, Ês7 / Noc7 each ±0.8 dB over UE Measurement bandwidth Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | Notes: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN Noc5 is the AWGN on frequency 5 Ês5 / Noc5 is the ratio of cell 5 signal / AWGN Noc6 is the AWGN on frequency 6 Ês6 / Noc6 is the ratio of cell 6 signal / AWGN Noc7 is the AWGN on frequency 7 Ês7 / Noc7 is the ratio of cell 7 signal / AWGN UE Measurement bandwidth: 5MHz Ch BW: PRBs 10-15 10MHz Ch BW: PRBs 22-27 20MHz Ch BW: PRBs 47-52 Ts = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] |
| 9.3.1 E-UTRAN FDD - UTRA FDD CPICH RSCP absolute accuracy | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior / Ioc ±0.3 dB CPICH Ec / Ior ±0.1 dB | Notes: Noc is the AWGN on cell 1 (E-UTRA )frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 (UTRA) frequency Ior / Ioc is the ratio of cell 2 signal / AWGN CPICH Ec / Ior is the fraction of cell 2 power assigned to the CPICH Physical channel |
| 9.3.2 E-UTRAN TDD - UTRA FDD CPICH RSCP absolute accuracy | Same as 9.3.1 |  |
| 9.3.3 E-UTRAN FDD - UTRA FDD CPICH RSCP absolute accuracy for 5MHz bandwidth | Same as 9.3.1 |  |
| 9.4.1 E-UTRAN FDD - UTRA FDD CPICH Ec/No absolute accuracy | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior / Ioc ±0.3 dB CPICH Ec / Ior ±0.1 dB | Notes: Noc is the AWGN on cell 1 (E-UTRA )frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 (UTRA) frequency Ior / Ioc is the ratio of cell 2 signal / AWGN CPICH Ec / Ior is the fraction of cell 2 power assigned to the CPICH Physical channel |
| 9.4.2 E-UTRAN TDD - UTRA FDD CPICH Ec/No absolute accuracy | Same as 9.4.1 | Same as 9.4.1 |
| 9.4.3 E-UTRAN FDD - UTRA FDD CPICH Ec/No absolute accuracy for 5MHz bandwidth | Same as 9.4.1 | Same as 9.4.1 |
| 9.5.1 E-UTRAN FDD - UTRA TDD P-CCPCH RSCP absolute accuracy | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior / Ioc ±0.3 dB PCCPCH EC/Ior ±0.1 dB DwPCH_EC/Ior ±0.1 dB | Notes: Noc is the AWGN on cell 1 (E-UTRA )frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 (UTRA) frequency Ior / Ioc is the ratio of cell 2 signal / AWGN PCCPCH Ec / Ior is the fraction of cell 2 power assigned to the PCCPCH Physical channel DwPCH_Ec / Ior is the fraction of cell 2 power assigned to the DwPCH channel |
| 9.5.2 E-UTRAN TDD - UTRA TDD P-CCPCH RSCP absolute accuracy | Same as 9.5.1 |  |
| 9.6.1 GSM RSSI accuracy for E-UTRAN FDD | E-UTRA Cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig GSM cell BCCH1 Signal level ±0.7 dB GSM cell BCCH 2 to 6 Signal level ±2.0 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN GSM cells BCCH 1 to 6 have only the wanted signal, without AWGN |
| 9.6.2 GSM RSSI accuracy for E-UTRAN TDD | E-UTRA Cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig GSM cell BCCH1 Signal level ±0.7 dB GSM cell BCCH 2 to 6 Signal level ±2.0 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN GSM cells BCCH 1 to 6 have only the wanted signal, without AWGN |
| 9.9.1.1 FDD Intra Frequency Serving Cell Absolute RSRP Accuracy | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN |
| 9.9.1.1_1 FDD Intra Frequency Serving Cell Absolute RSRP Accuracy (Rel 12 and forward) | Same as 9.9.1.1 | Same as 9.9.1.1 |
| 9.9.1.2 FDD Intra Frequency Serving Cell Absolute RSRQ Accuracy | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN |
| 9.9.2.1 TDD Intra Frequency Serving Cell Absolute RSRP Accuracy | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN |
| 9.9.2.1_1 TDD Intra Frequency Serving Cell Absolute RSRP Accuracy (Rel 12 and forward) | Same as 9.9.2.1 | Same as 9.9.2.1 |
| 9.9.2.2 TDD Intra Frequency Serving Cell Absolute RSRQ Accuracy | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN |
| 9.11.1 FS3 average RSSI accuracy case (PCell using FDD) | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for measurement PRBs Noc2 ±1.5 dB averaged over BWConfig Noc2 ±1.8 dB for measurement PRBs Ês1 / Noc1 and Ês2 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 and Ês2 / Noc2 each ±0.8 dB for measurement PRBs | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.11.2 FS3 average RSSI accuracy case (PCell using TDD) | Same as 9.11.1 | Same as 9.11.1 |
| 9.12.1 FS3 channel occupancy test (PCell using FDD) | Same as 9.11.1 | Same as 9.11.1 |
| 9.12.2 FS3 channel occupancy test (PCell using TDD) | Same as 9.11.1 | Same as 9.11.1 |
| 10.2 E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication | ± 0.6dB | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 10.4 E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication | ± 0.6dB | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 11.1 V2V UE Transmission Timing Accuracy Test | ±3Ts Uplink signal transmit timing relative to GNSS synchronization source Absolute GNSS signal level, ±1 dB | NOTE: TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] Same value as the A-GNSS Time Assistance tests in clause 5 of TS 37.571-1 |
| 11.2 Interruptions due to V2V sidelink communication | Noc ±3.0 dB averaged over BWConfig Ês / Noc ±0.3 dB Absolute GNSS signal level, ±1 dB | NOTE: Ês / Noc is the ratio of cell 1 signal / AWGN Same value as the A-GNSS Time Assistance tests in clause 5 of TS 37.571-1 |
| 12.1.1 V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference | synchronization source Noc ±3.0 dB averaged over BWConfig Ês / Noc ±0.3 dB ±3Ts Uplink signal transmit timing relative to GNSS | NOTE: Ês / Noc is the ratio of cell 1 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 12.1.2 V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference | Noc ±3.0 dB averaged over BWConfig Ês / Noc ±0.3 dB ±3Ts Uplink signal transmit timing relative to downlink | NOTE: Ês / Noc is the ratio of cell 1 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 12.2.1 Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference | Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês / Noc is the ratio of cell 1 signal / AWGN |
| 12.2.2 Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference | Noc ±1.5 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês / Noc is the ratio of SyncRef UE signal / AWGN |
| 12.3.1 V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority | Noc2 ±1.0 dB averaged over BWConfig SCH Ês1 / Noc ±0.3 dB averaged over BWConfig SCH Ês2 / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of SyncRef UE 1 signal / AWGN Ês2 / Noc is the ratio of SyncRef UE 2 signal / AWGN |
| 12.3.2 V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority | Noc2 ±1.0 dB averaged over BWConfig SCH Ês1 / Noc ±0.3 dB averaged over BWConfig SCH Ês2 / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of SyncRef UE 1 signal / AWGN Ês2 / Noc is the ratio of SyncRef UE 2 signal / AWGN |
| 12.4 Congestion Control Measurement Test for V2X UE | Noc ±1.5 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês / Noc is the ratio of Active UE signal / AWGN |
| 12.5 Interruptions due to V2X Sidelink Communication | Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês / Noc is the ratio of Active Sidelink UE signal / AWGN |
| 12.6.1 V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements | Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês / Noc is the ratio of Active Sidelink UE signal / AWGN |
| 12.6.2 V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements | Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês / Noc is the ratio of Active Sidelink UE signal / AWGN |
| 13.1.1.1 HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage | Noc1 ±1.0 dB averaged over BWConfig Êsn1 / Noc1 ±0.3 dB averaged over BWConfig Êsn2 / Noc1 ±0.3 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on frequency 1 Êsn1 / Noc1 is the ratio of nCell1 signal / AWGN Êsn2 / Noc1 is the ratio of nCell2 signal / AWGN |
| 13.1.1.1_1 HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage for GSO and NGSO | Same as 13.1.1.1 | Same as 13.1.1.1 |
| 13.1.1.2 HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with serving cell RRM measurement relaxation | Same as 13.1.1.1 | Same as 13.1.1.1 |
| 13.1.1.2_1 HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with serving cell RRM measurement relaxation for GSO and NGSO | Same as 13.1.1.1 | Same as 13.1.1.1 |
| 13.1.1.3 HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX | Same as 13.1.1.1 | Same as 13.1.1.1 |
| 13.1.1.3_1 FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX for GSO and NGSO | Same as 13.1.1.1 | Same as 13.1.1.1 |
| 13.1.1.4 HD – FDD Inter frequency case for UE Category NB1 Standalone mode in normal coverage | Same as 13.1.1.1 | Same as 13.1.1.1 |
| 13.1.1.5 HD – FDD Intra frequency case for UE Category NB1 Standalone mode in enhanced coverage, location-based cell reselection for NGSO | Same as 13.1.1.1 | Same as 13.1.1.1 |
| 13.1.1.6 FDD Inter frequency case for UE Category NB1 Standalone mode in enhanced coverage, time-based cell reselection for NGSO | Same as 13.1.1.1 | Same as 13.1.1.1 |
| 13.3.1.1 HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage | Noc ±1.0 dB averaged over BWConfig Êsn1 / Noc ±0.3 dB averaged over BWConfig Êsn2 / Noc ±0.3 dB averaged over BWConfig | NOTE: Êsn1 / Noc is the ratio of nCell 1 signal / AWGN Êsn2 / Noc is the ratio of nCell 2 signal / AWGN |
| 13.3.1.1_1 HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage for GSO and NGSO | Same as 13.3.1.1 | Same as 13.3.1.1 |
| 13.3.1.2 HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage | Same as 13.3.1.1 | Same as 13.3.1.1 |
| 13.3.1.2_1 HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage for GSO and NGSO | Same as 13.3.1.1 | Same as 13.3.1.1 |
| 13.3.1.3 HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage | 13.3.1.3 HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage | 13.3.1.3 HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage |
| 13.3.2.1 Contention Based Random Access Test for UE category NB1 UEs Standalone mode in normal coverage | Same as 6.2.16 | Same as 6.2.16 |
| 13.3.2.2 Contention Based Random Access Test for UE category NB1 UEs Standalone mode in Enhanced Coverage | Same as 6.2.16 | Same as 6.2.16 |
| 13.3.2.3 Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs Standalone mode in Enhanced Coverage | Same as 6.2.16 | Same as 6.2.16 |
| 13.4.1.1 E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under normal coverage for Satellite Access | Noc ±3.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig ±3Ts Uplink signal transmit timing relative to downlink ±0.5Ts Relative UE timing adjustment | NOTE: Ês / Noc is the ratio of active cell signal / AWGN The timing unit TS = 1/(15000 * 2048) seconds, as defined in 3GPP TS 36.211 [9] |
| 13.4.1.2 E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage for Satellite Access | Noc ±3.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig ±3Ts Uplink signal transmit timing relative to downlink | Same as 13.4.1.1 |
| 13.4.1.3 E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage with segment transmission in NGSO for Satellite Access | Same as 13.4.1.2 | Same as 13.4.1.1 |
| 13.4.2.1 HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage for Satellite Access | Noc ±3.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig Timing Advance Adjustment: ±0.5Ts | NOTE: Ês / Noc is the ratio of active cell signal / AWGN The timing unit TS = 1/(15000 * 2048) seconds, as defined in 3GPP TS 36.211 [9] |
| 13.4.2.2 HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhance Coverage for Satellite Access | Same as 13.4.2.1 | Same as 13.4.2.1 |
| 13.4.3.1 HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in normal coverage | Noc ±3.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês / Noc is the ratio of active cell signal / AWGN |
| 13.4.3.2 HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in enhanced coverage | Same as 13.4.3.1 | Same as 13.4.3.1 |
| 13.4.3.3 HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 Standalone mode in Enhanced Coverage | Noc ±3.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês / Noc is the ratio of active cell signal / AWGN |
| 13.4.3.4 HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 Standalone mode in Normal Coverage | Same as 13.4.3.3 | Same as 13.4.3.3 |
| 13.4.3.5 HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage | Same as 13.4.3.3 | Same as 13.4.3.3 |
| 13.4.3.6 HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 Standalone mode in Enhanced Coverage | Same as 13.4.3.3 | Same as 13.4.3.3 |
| 13.4.3.7 HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage | Same as 13.4.3.1 | Same as 13.4.3.1 |
| 13.4.3.8 HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Enhanced Coverage | Same as 13.4.3.1 | Same as 13.4.3.1 |
| 13.5.1 HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access | Noc ±1.0 dB averaged over BWConfig Êsn1 / Noc ±0.3 dB averaged over BWConfig Êsn2 / Noc ±0.3 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on frequency 1 Êsn1 / Noc1 is the ratio of nCell1 signal / AWGN Êsn2 / Noc1 is the ratio of nCell2 signal / AWGN |
| 13.5.2 HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access | Same as 13.5.1 | Same as 13.5.1 |
| 13.5.3 HD-FDD Intra-frequency location-based neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access | Same as 13.5.1 | Same as 13.5.1 |
| 13.6.2.1 HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage | ± 0.6dB | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 13.6.2.2 HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage | Same as 13.6.2.1 | Same as 13.6.2.1 |
| 13.6.2.3 HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage | Same as 13.6.2.1 | Same as 13.6.2.1 |
| 13.6.2.4 HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage | Same as 13.6.2.1 | Same as 13.6.2.1 |
| 13.6.2.5 HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under normal coverage | Same as 13.6.2.1 | Same as 13.6.2.1 |
| 13.6.2.6 HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under enhanced coverage | Same as 13.6.2.1 | Same as 13.6.2.1 |
| In addition, the following Test System uncertainties and related constraints apply. Any additional constraints are defined in the specific tests. |  |  |
| AWGN Bandwidth |  | ≥ 1.08MHz, 2.7MHz, 4.5MHz, 9MHz, 13.5MHz, 18MHz; NRB x 180kHz according to BWConfig |
| AWGN absolute power uncertainty |  | Test-specific |
| AWGN flatness and signal flatness, max deviation for any Resource Block, relative to average over BWConfig |  | ±2 dB |
| AWGN peak to average ratio |  | ≥10 dB @0.001% |
| Signal-to noise ratio uncertainty |  | Test-specific |
| Fading profile power uncertainty |  | ±0.5 dB |
| Fading profile delay uncertainty, relative to frame timing |  | ±5 ns (excludes absolute errors related to baseband timing) |

# F.2	Interpretation of measurement results (normative)

See 3GPP TS 36.521-1 [10] Annex F2.

# F.3	Test Tolerance and Derivation of Test Requirements (informative)

## F.3.0	General

See 3GPP TS 36.521-1 [10] Annex F3.

## F.3.1	Measurement of test environments

See 3GPP TS 36.521-1 [10] Annex F3.1.

## F.3.2	Measurement of RRM requirements

Because the relationships between the Test system uncertainties and the Test Tolerances are often complex, it is not always possible to give a simple derivation of the Test Requirement in this document. The analysis is recorded in 3GPP TR 36 903 [20].

Table F.3.2-1: Derivation of Test Requirements (RRM tests)

| Test |  |  | Minimum Requirement in 3GPP TS 36.133 |  |  | Test Tolerance (TT) |  |  |  |  |  |  | Test Requirement in 3GPP TS 36.521-3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4.2.1 E-UTRA FDD - FDD cell re-selection intra frequency |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13.00dB Ês2 / Noc: +16.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +16.00dB Ês2 / Noc: +13.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.45dB During T3: 0dB +0.45dB 0dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13.00dB Ês2 / Noc: +16.45dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +16.45dB Ês2 / Noc: +13.00dB |  |
| 4.2.2 E-UTRA TDD - TDD cell re-selection intra frequency |  |  | Same as 4.2.1 |  |  | Same as 4.2.1 |  |  |  |  |  |  | Same as 4.2.1 |  |
| 4.2.3 E-UTRA FDD - FDD cell re-selection inter frequency |  |  | During T0: Noc1: -98dBm/15kHz Ês1 / Noc1: -4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +14.00dB During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -4.00dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: +14.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +12.00dB |  |  | During T0: -1.35dB +0.3dB -1.35dB +0.4dB During T1: -1.35dB +2.4dB -1.35dB +0.3dB During T2: -1.35dB +2.4dB -1.35dB 0dB During T3: -1.35dB +2.4dB -1.35dB +2.4dB |  |  |  |  |  |  | During T0: Noc1: -99.35dBm/15kHz Ês1 / Noc1: -3.70dB Noc2: -99.35dBm/15kHz Ês2 / Noc2: +14.40dB During T1: Noc1: -99.35dBm/15kHz Ês1 / Noc1: +16.40dB Noc2: -99.35dBm/15kHz Ês2 / Noc2: -3.70dB During T2: Noc1: -99.35dBm/15kHz Ês1 / Noc1: +16.40dB Noc2: -99.35dBm/15kHz Ês2 / Noc2: -infinity During T3: Noc1: -99.35dBm /15kHz Ês1 / Noc1: +16.40dB Noc2: -99.35dBm/15kHz Ês2 / Noc2: +14.40dB |  |
| 4.2.4 E-UTRA FDD - TDD cell re-selection inter frequency |  |  | Same as 4.2.3 |  |  | Same as 4.2.3 |  |  |  |  |  |  | Same as 4.2.3 |  |
| 4.2.5 E-UTRA TDD - FDD cell re-selection inter frequency |  |  | Same as 4.2.3 |  |  | Same as 4.2.3 |  |  |  |  |  |  | Same as 4.2.3 |  |
| 4.2.6 E-UTRA TDD - TDD cell re-selection inter frequency |  |  | Same as 4.2.3 |  |  | Same as 4.2.3 |  |  |  |  |  |  | Same as 4.2.3 |  |
| 4.2.7 E-UTRA FDD Inter frequency re-selection in the existence of non-allowed CSG cell |  |  | During T0: Noc1: -98dBm/15kHz Ês1 / Noc1: +13.00dB Ês3 / Noc1: -infinity Noc2: -98dBm/15kHz Ês2 / Noc2: -3.00dB During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +8.00dB Ês3 / Noc1: +8.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +8.00dB Ês3 / Noc1: +13.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +13.00dB During T3: Noc1: -98dBm/15kHz Ês1 / Noc1: +13.00dB Ês3 / Noc1: +38.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +8.00dB |  |  | During T0: 0dB 0dB 0dB 0dB 0dB During T1: 0dB -0.2dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T0: Noc1: -98dBm/15kHz Ês1 / Noc1: +13.00dB Ês3 / Noc1: -infinitydB Noc2: -98dBm/15kHz Ês2 / Noc2: -3.00dB During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +7.80dB Ês3 / Noc1: +8.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +8.00dB Ês3 / Noc1: +13.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +13.00dB During T3: Noc1: -98dBm/15kHz Ês1 / Noc1: +13.00dB Ês3 / Noc1: +38.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +8.00dB |  |
| 4.2.8 E-UTRA TDD Inter frequency re-selection in the existence of non-allowed CSG cell |  |  | Same as 4.2.7 |  |  | Same as 4.2.7 |  |  |  |  |  |  | Same as 4.2.7 |  |
| 4.2.9 E-UTRAN FDD - FDD intra frequency cell re-selection case for 5MHz bandwidth |  |  | Same as 4.2.1 |  |  | Same as 4.2.1 |  |  |  |  |  |  | Same as 4.2.1 |  |
| 4.2.10 E-UTRAN FDD - FDD reselection using an increased number of carriers |  |  | During T0: Noc1: -98dBm/15kHz Ês1 / Noc1: +14.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +8.00dB Noc3: -98dBm/15kHz Ês3 / Noc3: +8.00dB Noc4: -98dBm/15kHz Ês4 / Noc4: +8.00dB During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +8.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +14.00dB Noc3: -98dBm/15kHz Ês3 / Noc3: +8.00dB Noc4: -98dBm/15kHz Ês4 / Noc4: +8.00dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +8.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +8.00dB Noc3: -98dBm/15kHz Ês3 / Noc3: +14.00dB Noc4: -98dBm/15kHz Ês4 / Noc4: +8.00dB During T3: Noc1: -98dBm/15kHz Ês1 / Noc1: +8.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +8.00dB Noc3: -98dBm/15kHz Ês3 / Noc3: +8.00dB Noc4: -98dBm/15kHz Ês4 / Noc4: +14.00dB During T4: Noc1: -98dBm/15kHz Ês1 / Noc1: +14.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +8.00dB Noc3: -98dBm/15kHz Ês3 / Noc3: +8.00dB Noc4: -98dBm/15kHz Ês4 / Noc4: +8.00dB |  |  | During T0: -1.15dB +1.10dB -1.15dB 0dB -1.15dB 0dB -1.15dB 0dB During T1: -1.15dB 0dB -1.15dB +1.10dB -1.15dB 0dB -1.15dB 0dB During T2: -1.15dB 0dB -1.15dB 0dB -1.15dB +1.10dB -1.15dB 0dB During T3: -1.15dB 0dB -1.15dB 0dB -1.15dB 0dB -1.15dB +1.10dB During T4: -1.15dB +1.10dB -1.15dB 0dB -1.15dB 0dB -1.15dB 0dB |  |  |  |  |  |  | During T0: Noc1: -99.15dBm/15kHz Ês1 / Noc1: +15.10dB Noc2: -99.15dBm/15kHz Ês2 / Noc2: +8.00dB Noc3: -99.15dBm/15kHz Ês3 / Noc3: +8.00dB Noc4: -99.15dBm/15kHz Ês4 / Noc4: +8.00dB During T1: Noc1: -99.15dBm/15kHz Ês1 / Noc1: +8.00dB Noc2: -99.15dBm/15kHz Ês2 / Noc2: +15.10dB Noc3: -99.15dBm/15kHz Ês3 / Noc3: +8.00dB Noc4: -99.15dBm/15kHz Ês4 / Noc4: +8.00dB During T2: Noc1: -99.15dBm/15kHz Ês1 / Noc1: +8.00dB Noc2: -99.15dBm/15kHz Ês2 / Noc2: +8.00dB Noc3: -99.15dBm/15kHz Ês3 / Noc3: +15.10dB Noc4: -99.15dBm/15kHz Ês4 / Noc4: +8.00dB During T3: Noc1: -99.15dBm/15kHz Ês1 / Noc1: +8.00dB Noc2: -99.15dBm/15kHz Ês2 / Noc2: +8.00dB Noc3: -99.15dBm/15kHz Ês3 / Noc3: +8.00dB Noc4: -99.15dBm/15kHz Ês4 / Noc4: +15.10dB During T4: Noc1: -99.15dBm/15kHz Ês1 / Noc1: +15.10dB Noc2: -99.15dBm/15kHz Ês2 / Noc2: +8.00dB Noc3: -99.15dBm/15kHz Ês3 / Noc3: +8.00dB Noc4: -99.15dBm/15kHz Ês4 / Noc4: +8.00dB |  |
| 4.2.11 E-UTRAN TDD - TDD reselection using an increased number of carriers |  |  | Same as 4.2.10 |  |  | Same as 4.2.10 |  |  |  |  |  |  | Same as 4.2.10 |  |
| 4.2.12 E-UTRAN FDD - FDD Intra frequency case for Cat-M1 UE in normal coverage |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +12.00dB Ês2 / Noc: +16.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +16.00dB Ês2 / Noc: +12.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.45dB During T3: 0dB +0.45dB 0dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +12.00dB Ês2 / Noc: +16.45dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +16.45dB Ês2 / Noc: +12.00dB |  |
| 4.2.13 E-UTRAN HD - FDD Intra frequency case for Cat-M1 UE in normal coverage |  |  | Same as 4.2.12 |  |  | Same as 4.2.12 |  |  |  |  |  |  | Same as 4.2.12 |  |
| 4.2.14 E-UTRA TDD - TDD intra frequency case for Cat-M1 UE in normal coverage |  |  | Same as 4.2.12 |  |  | Same as 4.2.12 |  |  |  |  |  |  | Same as 4.2.12 |  |
| 4.2.15 E-UTRAN FDD - FDD Intra frequency case for Cat-M1 UE in enhanced coverage |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: -7.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -12.00dB Ês2 / Noc: -7.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: -7.00dB Ês2 / Noc: -12.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.45dB During T3: 0dB +0.45dB 0dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: -7.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -12.00dB Ês2 / Noc: -6.55dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: -6.55dB Ês2 / Noc: -12.00dB |  |
| 4.2.16 E-UTRAN HD - FDD Intra frequency case for Cat-M1 UE in enhanced coverage |  |  | Same as 4.2.15 |  |  | Same as 4.2.15 |  |  |  |  |  |  | Same as 4.2.15 |  |
| 4.2.17 E-UTRAN TDD - TDD Intra frequency case for Cat-M1 UE in enhanced coverage |  |  | Same as 4.2.15 |  |  | Same as 4.2.15 |  |  |  |  |  |  | Same as 4.2.15 |  |
| 4.2.18 HD-FDD Intra frequency cell reselection for Category NB1 UE in In-Band Mode under Normal Coverage |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Êsn1 / Noc: 17.00dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Êsn1 / Noc: 13.00dB Êsn2 / Noc: 17.00dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Êsn1 / Noc: 17.00dB Êsn2 / Noc: 13.00dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB +0.45dB During T3: 0dB 0dB +0.45dB 0dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Êsn1 / Noc: 17.00dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Êsn1 / Noc: 13.00dB Êsn2 / Noc: 17.45dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Êsn1 / Noc: 17.45dB Êsn2 / Noc: 13.00dB |  |
| 4.2.19 HD - FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Ês2 / Noc: 3.00dB Êsn1 / Noc: -9.00dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Ês2 / Noc: 3.00dB Êsn1 / Noc: -9.00dB Êsn2 / Noc: -0.70dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Ês2 / Noc: 3.00dB Êsn1 / Noc: -0.70dB Êsn2 / Noc: -9.00dB |  |  | During T1: 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0.45dB During T3: 0dB 0dB 0dB 0.45dB 0dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Ês2 / Noc: 3.00dB Êsn1 / Noc: -9.00dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Ês2 / Noc: 3.00dB Êsn1 / Noc: -9.00dB Êsn2 / Noc: -0.25dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Ês2 / Noc: 3.00dB Êsn1 / Noc: -0.25dB Êsn2 / Noc: -9.00dB |  |
| 4.2.20 E-UTRAN FDD - FDD Intra frequency case for UE Category 1bis |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +17.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13.00dB Ês2 / Noc: +17.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +17.00dB Ês2 / Noc: +13.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.45dB During T3: 0dB +0.45dB 0dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +17.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13.00dB Ês2 / Noc: +17.45dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +17.45dB Ês2 / Noc: +13.00dB |  |
| 4.2.21 E-UTRAN TDD - TDD Intra frequency case for UE Category 1bis |  |  | Same as 4.2.21 |  |  | Same as 4.2.21 |  |  |  |  |  |  | Same as 4.2.21 |  |
| 4.2.22 E-UTRAN FDD - FDD cell re-selection intra frequency case for UE configured with highSpeedEnhancedMeasFlag |  |  | Same as 4.2.1 |  |  | Same as 4.2.1 |  |  |  |  |  |  | Same as 4.2.1 |  |
| 4.2.23 E-UTRAN TDD - TDD cell re-selection intra frequency case for UE configured with highSpeedEnhancedMeasFlag |  |  | Same as 4.2.1 |  |  | Same as 4.2.1 |  |  |  |  |  |  | Same as 4.2.1 |  |
| 4.2.24 HD - FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Êsn1 / Noc: -9.00dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Êsn1 / Noc: -9.00dB Êsn2 / Noc: -0.70dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Êsn1 / Noc: -0.70dB Êsn2 / Noc: -9.00dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0.45dB During T3: 0dB 0dB 0.45dB 0dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Êsn1 / Noc: -9.00dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Êsn1 / Noc: -9.00dB Êsn2 / Noc: -0.25dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Êsn1 / Noc: -0.25dB Êsn2 / Noc: -9.00dB |  |
| 4.2.28 E-UTRAN FDD - FDD Inter frequency case for CE UE in enhanced coverage |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: -5.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -12.00dB Ês2 / Noc: -5.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: -5.00dB Ês2 / Noc: -12.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.5dB During T3: 0dB 0.5dB 0dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: -5.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -12.00dB Ês2 / Noc: -4.50dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: -4.50dB Ês2 / Noc: -12.00dB |  |
| 4.2.29 E-UTRAN HD - FDD Inter frequency case for CE UE in enhanced coverage |  |  | Same as 4.2.28 |  |  | Same as 4.2.28 |  |  |  |  |  |  | Same as 4.2.28 |  |
| 4.2.30 E-UTRAN TDD Inter frequency case for CE UE in enhanced coverage |  |  | Same as 4.2.28 |  |  | Same as 4.2.28 |  |  |  |  |  |  | Same as 4.2.28 |  |
| 4.2.35 E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage |  |  | Same as 4.2.18 |  |  | Same as 4.2.18 |  |  |  |  |  |  | Same as 4.2.18 |  |
| 4.2.36 E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage |  |  | Same as 4.2.19 |  |  | Same as 4.2.19 |  |  |  |  |  |  | Same as 4.2.19 |  |
| 4.2.37 E-UTRAN TDD - TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage |  |  | Same as 4.2.24 |  |  | Same as 4.2.24 |  |  |  |  |  |  | Same as 4.2.24 |  |
| 4.2.38 HD - FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Êsn1 / Noc: 23.00dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Êsn1 / Noc: 13.00dB Êsn2 / Noc: 17.00dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB +0.45dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Êsn1 / Noc: 23.00dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Êsn1 / Noc: 13.00dB Êsn2 / Noc: 17.45dB |  |
| 4.3.1.1 E-UTRA FDD - UTRAN FDD cell reselection: UTRA FDD is of higher priority |  |  | During T1: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +14.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: -∞dB CPICH_Ec/Ior: -10.00dB During T2: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +14.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.00dB CPICH_Ec/Ior: -10.00dB During T3: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +14.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: -5.00dB CPICH_Ec/Ior: -10.00dB |  |  | During T1: 0dB +1.05dB -0.1dB 0dB 0dB During T2: 0dB +1.05dB -0.1dB +0.9dB 0dB During T3: 0dB +1.05dB -0.1dB -0.7dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +1.05dB UTRA Cell 2 Ioc: -70.10dBm/3.84MHz Ior / Ioc: -∞dB CPICH_Ec/Ior: -10.00dB During T2: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +1.05dB UTRA Cell 2 Ioc: -70.10dBm/3.84MHz Ior / Ioc: +11.90dB CPICH_Ec/Ior: -10.00dB During T3: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +1.05dB UTRA Cell 2 Ioc: -70.10dBm/3.84MHz Ior / Ioc: -5.70dB CPICH_Ec/Ior: -10.00dB |  |
| 4.3.1.2 E-UTRAN FDD - UTRAN FDD cell re-selection: UTRA FDD is of lower priority |  |  | During T1: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +12.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +13.00dB CPICH_Ec/Ior: -10.00dB During T2: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: -4.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +13.00dB CPICH_Ec/Ior: -10.00dB |  |  | During T1: -1.35dB +2.40dB 0dB +0.80dB 0dB During T2: -1.35dB +0.30dB 0dB +0.80dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -99.35dBm/15kHz Ês / Noc: +14.40dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +13.80dB CPICH_Ec/Ior: -10.00dB During T2: E-UTRA Cell 1 Noc: -99.35dBm/15kHz Ês / Noc: -3.70dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +13.80dB CPICH_Ec/Ior: -10.00dB |  |
| 4.3.1.3 EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority |  |  | During T1, T2: E-UTRA Cell 1 Noc: -104.00dBm/15kHz Ês / Noc: +22.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +13.00dB CPICH_Ec/Ior: -10.00dB During T3, T4: E-UTRA Cell 1 Noc: -104.00dBm/15kHz Ês / Noc: -3.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +13.00dB CPICH_Ec/Ior: -10.00dB |  |  | During T1,T2 0dB 0dB 0dB +0.80dB 0dB During T3,T4 0dB 0dB 0dB +0.80dB 0dB |  |  |  |  |  |  | During T1, T2: E-UTRA Cell 1 Noc: -104.00dBm/15kHz Ês / Noc: +22.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +13.80dB CPICH_Ec/Ior: -10.00dB During T3, T4: E-UTRA Cell 1 Noc: -104.00dBm/15kHz Ês / Noc: -3.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +13.80dB CPICH_Ec/Ior: -10.00dB |  |
| 4.3.1.4 E-UTRAN FDD - UTRAN FDD cell re-selection: UTRA FDD is of lower priority for 5MHz bandwidth |  |  | Same as 4.3.1.2 |  |  |  |  |  |  |  |  |  |  |  |
| 4.3.1.5 Idle mode FDD to UTRA FDD interRAT reselection |  |  | During T0: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +14.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.00dB CPICH_Ec/Ior: -10.00dB UTRA Cell 3 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.00dB CPICH_Ec/Ior: -10.00dB During T1: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: -4.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.00dB CPICH_Ec/Ior: -10.00dB UTRA Cell 3 Ioc: -70.00dBm/3.84MHz Ior / Ioc: -5.00dB CPICH_Ec/Ior: -10.00dB During T2 E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: -4.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.00dB CPICH_Ec/Ior: -10.00dB UTRA Cell 3 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.00dB CPICH_Ec/Ior: -10.00dB During T3 E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: -4.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: -5.00dB CPICH_Ec/Ior: -10.00dB UTRA Cell 3 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.00dB CPICH_Ec/Ior: -10.00dB During T4 E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: -4.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.00dB CPICH_Ec/Ior: -10.00dB UTRA Cell 3 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.00dB CPICH_Ec/Ior: -10.00dB |  |  | During T0: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T1: 0dB 0.3dB 0dB 0.8dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0.8dB 0dB 0dB 0dB 0dB During T3: 0dB 0.3dB 0dB 0dB 0dB 0dB 0.8dB 0dB During T4: 0dB 0dB 0dB 0dB 0dB 0dB 0.8dB 0dB |  |  |  |  |  |  | During T0: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +14.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.00dB CPICH_Ec/Ior: -10.00dB UTRA Cell 3 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.00dB CPICH_Ec/Ior: -10.00dB During T1: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: -3.70dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.80dB CPICH_Ec/Ior: -10.00dB UTRA Cell 3 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.00dB CPICH_Ec/Ior: -10.00dB During T2 E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: -4.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.80dB CPICH_Ec/Ior: -10.00dB UTRA Cell 3 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.00dB CPICH_Ec/Ior: -10.00dB During T3 E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: -3.70dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: -5.00dB CPICH_Ec/Ior: -10.00dB UTRA Cell 3 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.80dB CPICH_Ec/Ior: -10.00dB During T4 E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: -4.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.00dB CPICH_Ec/Ior: -10.00dB UTRA Cell 3 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.80dB CPICH_Ec/Ior: -10.00dB |  |
| 4.3.2 E-UTRA FDD - UTRAN TDD cell re-selection |  |  | During T1: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: +11.00dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: +11.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: -3.0dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: +11.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |  | During T1: 0dB 0.1dB 0dB 0dB 0dB 0dB During T2: 0dB 0.1dB 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: +11.10dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: +11.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: -3.1dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: +11.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |
| 4.3.2A E-UTRA FDD to UTRA TDD cell re-selection for IncMon |  |  | During T0: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: +11.00dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB UTRA Cell 3 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T1: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: -3.00dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: +11.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB UTRA Cell 3 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: +11.00dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB UTRA Cell 3 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T3: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: -3.00dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB UTRA Cell 3 Ioc: -80.0dBm/1.28MHz Îor / Ioc: +11.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T04 E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: +11.00dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB UTRA Cell 3 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |  | During T0: 0dB +0.5dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T1: 0dB -0.5dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB +0.5dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T3: 0dB -0.5dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T4: 0dB +0.5dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T0: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: +11.05dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB UTRA Cell 3 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T1: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: -3.05dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: +11.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB UTRA Cell 3 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: +11.05dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB UTRA Cell 3 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T3: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: -3.05dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: +11.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB UTRA Cell 3 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T4: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: +11.05dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB UTRA Cell 3 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |
| 4.3.3 E-UTRAN TDD - UTRAN FDD cell re-selection: UTRA FDD is of lower priority |  |  | Same as 4.3.1.2 |  |  |  |  | Same as 4.3.1.2 |  |  |  |  | Same as 4.3.1.2 |  |
| 4.3.3A Idle mode TDD to UTRA FDD interRAT reselection |  |  | Same as 4.3.1.5 |  |  |  |  | Same as 4.3.1.5 |  |  |  |  | Same as 4.3.1.5 |  |
| 4.3.4.1 E-UTRA TDD - UTRAN TDD cell re-selection : UTRA is of higher priority |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +11.00dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Îor / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm /15kHz Ês / Noc: +11.00dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Îor / Ioc: +11.00dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T3: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +11.00dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Îor / Ioc: -3.00dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |  | During T1: 0dB +0.1dB 0dB 0dB During T2: 0dB +0.1dB 0dB 0dB 0dB 0dB During T3: 0dB +0.1dB 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc: -98.0dBm/15kHz Ês / Noc: +11.1dB Ioc: -80.0dBm/1.28MHz Îor / Ioc: -infinity During T2: Noc: -98.0dBm /15kHz Ês / Noc: +11.1dB Ioc: -80.0dBm/1.28MHz Îor / Ioc: +11.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T3: Noc: -98.0dBm/15kHz Ês / Noc: +11.1dB Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |
| 4.3.4.2 E-UTRA TDD - UTRAN TDD cell re-selection : UTRA is of lower priority |  |  | Same as 4.3.2 |  |  | Same as 4.3.2 |  |  |  |  |  |  | Same as 4.3.2 |  |
| 4.3.4.3 EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority |  |  | During T1, T2: E-UTRA Cell 1 Noc: -104.0dBm/15kHz Ês / Noc: +22.00dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: +13.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T3, T4: E-UTRA Cell 1 Noc: -104.0dBm/15kHz Ês / Noc: -3.0dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: +13.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |  | During T1, T2: 0dB 0dB 0dB 0dB 0dB 0dB During T3, T4: 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1, T2: E-UTRA Cell 1 Noc: -104.0dBm/15kHz Ês / Noc: +22.00dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: +13.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T3, T4: E-UTRA Cell 1 Noc: -104.0dBm/15kHz Ês / Noc: -3.0dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: +13.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |
| 4.3.4.4 E-UTRA TDD to UTRA TDD cell re-selection for IncMon |  |  | Same as 4.3.2A |  |  | Same as 4.3.2A |  |  |  |  |  |  | Same as 4.3.2A |  |
| 4.4.1 E-UTRAN FDD - GSM cell re-selection |  |  | During T1: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +9.00dB GSM Cell 2 Signal level: -90.00dBm During T2: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: -4.00dB GSM Cell 2 Signal level: -75.0dBm |  |  | During T1: -1.4dB +1.5dB 0dB During T2: -1.4dB +0.3dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -99.40dBm/15kHz Ês / Noc: +10.5dB GSM Cell 2 Signal level: -90.00dBm During T2: E-UTRA Cell 1 Noc: -99.40dBm/15kHz Ês / Noc: -3.70dB GSM Cell 2 Signal level: -75.00dBm |  |
| 4.4.2 E-UTRAN TDD - GSM cell re-selection |  |  | Same as 4.4.1 |  |  | Same as 4.4.1 |  |  |  |  |  |  | Same as 4.4.1 |  |
| 4.5.1.1 RRC IDLE / E-UTRAN to HRPD Cell re-selection / E-UTRAN FDD - HRPD cell re-selection: HRPD is of lower priority |  |  | During T1: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +9.00dB HRPD Cell 2 Ioc: -55.00dBm/1.2288MHz Ior / Ioc: 0 dB During T2: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: -4.00dB HRPD Cell 2 Ioc: -55.00dBm/1.2288MHz Ior / Ioc: 0 dB |  |  | During T1 -1.4dB 1.5dB 0dB 0dB During T2 -1.4dB 0.3dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -99.4dBm/15kHz Ês / Noc: +10.5dB HRPD Cell 2 Ioc: -55.00dBm/1.2288MHz Ior / Ioc: 0 dB During T2: E-UTRA Cell 1 Noc: -99.4dBm/15kHz Ês / Noc: -3.70dB HRPD Cell 2 Ioc: -55.00dBm/1.2288MHz Ior / Ioc: 0 dB |  |
| 4.5.2.1 E-UTRAN TDD - HRPD Cell Reselection: HRPD is of Lower Priority |  |  | Same as 4.5.1.1 |  |  | Same as 4.5.1.1 |  |  |  |  |  |  | Same as 4.5.1.1 |  |
| 5.1.1 E-UTRAN FDD-FDD Handover intra frequency case |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.5dB During T3: 0dB 0dB +0.5dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.50dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00 Ês2 / Noc: +11.50dB |  |
| 5.1.2 E-UTRAN TDD-TDD Handover intra frequency case |  |  | Same as 5.1.1 |  |  | Same as 5.1.1 |  |  |  |  |  |  | Same as 5.1.1 |  |
| 5.1.3 E-UTRAN FDD-FDD Handover inter frequency case |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB +0.5dB During T3: 0dB 0dB 0dB +0.5dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.50dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.50dB |  |
| 5.1.3_2 E-UTRAN FDD - FDD Inter frequency handover for UE Category 1bis |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB |  |
| 5.1.4 E-UTRAN TDD-TDD Handover inter frequency case |  |  | Same as 5.1.3 |  |  | Same as 5.1.3 |  |  |  |  |  |  | Same as 5.1.3 |  |
| 5.1.4_2 E-UTRAN TDD - TDD Inter frequency handover for UE Category 1bis |  |  | Same as 5.1.3_2 |  |  | Same as 5.1.3_2 |  |  |  |  |  |  | Same as 5.1.3_2 |  |
| 5.1.5 E-UTRAN FDD-FDD inter-frequency Handover with unknown target cell |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB |  |
| 5.1.5_2 E-UTRAN FDD - FDD Inter frequency handover: unknown target cell for UE Category 1bis |  |  | Same as 5.1.5 |  |  | Same as 5.1.5 |  |  |  |  |  |  | Same as 5.1.5 |  |
| 5.1.6 E-UTRAN TDD-TDD inter-frequency Handover with unknown target cell |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +5.0dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +5.0dB |  |
| 5.1.6_2 E-UTRAN TDD - TDD Inter frequency handover; unknown Target Cell for UE Category 1bis |  |  | Same as 5.1.6 |  |  | Same as 5.1.6 |  |  |  |  |  |  | Same as 5.1.6 |  |
| 5.1.7 E-UTRAN FDD-TDD Handover inter frequency case |  |  | Same as 5.1.3 |  |  | Same as 5.1.3 |  |  |  |  |  |  | Same as 5.1.3 |  |
| 5.1.7_2 E-UTRAN FDD - TDD Inter frequency handover for UE Category 1bis |  |  | Same as 5.1.3_2 |  |  | Same as 5.1.3_2 |  |  |  |  |  |  | Same as 5.1.3_2 |  |
| 5.1.8 E-UTRAN TDD-FDD Handover inter frequency case |  |  | Same as 5.1.3 |  |  | Same as 5.1.3 |  |  |  |  |  |  | Same as 5.1.3 |  |
| 5.1.8_2 E-UTRAN TDD - FDD Inter frequency handover for UE Category 1bis |  |  | Same as 5.1.3_2 |  |  | Same as 5.1.3_2 |  |  |  |  |  |  | Same as 5.1.3_2 |  |
| 5.1.9 E-UTRAN FDD-FDD Intra frequency handover for 5MHz bandwidth |  |  | Same as 5.1.1 |  |  | Same as 5.1.1 |  |  |  |  |  |  | Same as 5.1.1 |  |
| 5.1.10 E-UTRAN FDD-FDD Handover intra frequency handover for UE category 0 |  |  | Same as 5.1.1 |  |  | Same as 5.1.1 |  |  |  |  |  |  | Same as 5.1.1 |  |
| 5.1.11 E-UTRAN HD-FDD Handover intra frequency handover for UE category 0 |  |  | Same as 5.1.1 |  |  | Same as 5.1.1 |  |  |  |  |  |  | Same as 5.1.1 |  |
| 5.1.12 E-UTRAN TDD-TDD Handover intra frequency handover for UE category 0 |  |  | Same as 5.1.1 |  |  | Same as 5.1.1 |  |  |  |  |  |  | Same as 5.1.1 |  |
| 5.1.13 E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +12.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +12.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.5dB During T3: 0dB 0dB +0.5dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +12.50dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +12.50dB |  |
| 5.1.14 E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA |  |  | Same as 5.1.13 |  |  | Same as 5.1.13 |  |  |  |  |  |  | Same as 5.1.13 |  |
| 5.1.15 E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA |  |  | Same as 5.1.13 |  |  | Same as 5.1.13 |  |  |  |  |  |  | Same as 5.1.13 |  |
| 5.1.16 E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: -12.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -12.00dB Ês2 / Noc: -7dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: -12.00dB Ês2 / Noc: -7dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.5dB During T3: 0dB 0dB +0.5dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: -12.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -12.00dB Ês2 / Noc: -6.5dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: -12.00 Ês2 / Noc: -6.5dB |  |
| 5.1.17 E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB |  |  | Same as 5.1.16 |  |  | Same as 5.1.16 |  |  |  |  |  |  | Same as 5.1.16 |  |
| 5.1.18 E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB |  |  | Same as 5.1.16 |  |  | Same as 5.1.16 |  |  |  |  |  |  | Same as 5.1.16 |  |
| 5.1.19 E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +12.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +12.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.5dB During T3: 0dB 0dB +0.5dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +12.50dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00 Ês2 / Noc: +12.50dB |  |
| 5.1.20 E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis |  |  | Same as 5.1.19 |  |  | Same as 5.1.19 |  |  |  |  |  |  | Same as 5.1.19 |  |
| 5.1.28 E-UTRAN HD-FDD inter frequency handover for CE UEs in CEModeA |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -3dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -3dB Noc2: -98dBm/15kHz Ês2 / Noc2: +4dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: -3dB Noc2: -98dBm/15kHz Ês2 / Noc2: +4dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -3dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -3dB Noc2: -98dBm/15kHz Ês2 / Noc2: +4dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: -3dB Noc2: -98dBm/15kHz Ês2 / Noc2: +4dB |  |
| 5.1.34 E-UTRAN HD-FDD intra frequency handover for CE UEs in CEModeA without SFN acquisition |  |  | Same as 5.1.14 The Test Tolerance values are taken from the analysis for similar HD-FDD Test case 5.1.14 and no separate analysis is needed. |  |  | Same as 5.1.14 The Test Tolerance values are taken from the analysis for similar HD-FDD Test case 5.1.14 and no separate analysis is needed. |  |  |  |  |  |  | Same as 5.1.14 The Test Tolerance values are taken from the analysis for similar HD-FDD Test case 5.1.14 and no separate analysis is needed. |  |
| 5.1.41 E-UTRAN FDD – FDD Intra-band Inter-frequency sync DAPS handover |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB During T4: Noc1: -98dBm /15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB During T5: Noc1: -98dBm /15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB +0.5dB During T3: 0dB 0dB 0dB +0.5dB During T4: 0dB 0dB 0dB +0.5dB During T5: 0dB 0dB 0dB +0.5dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.50dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.50dB During T4: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.50dB During T5: Noc1: -98dBm /15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.50dB |  |
| 5.1.42 E-UTRAN FDD – FDD Intra-band Inter-frequency async DAPS handover |  |  | Same as 5.1.41 |  |  | Same as 5.1.41 |  |  |  |  |  |  | Same as 5.1.41 |  |
| 5.1.43 E-UTRAN FDD – FDD Inter-band Inter-frequency sync DAPS handover |  |  | Same as 5.1.41 |  |  | Same as 5.1.41 |  |  |  |  |  |  | Same as 5.1.41 |  |
| 5.1.44 E-UTRAN FDD – FDD Inter-band Inter-frequency async DAPS handover |  |  | Same as 5.1.41 |  |  | Same as 5.1.41 |  |  |  |  |  |  | Same as 5.1.41 |  |
| 5.1.45 E-UTRAN FDD - FDD Intra frequency DAPS handover |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB During T5: Noc: -98dBm /15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.5dB During T3: 0dB 0dB +0.5dB During T4: 0dB 0dB +0.5dB During T5: 0dB 0dB +0.5dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.50dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00 Ês2 / Noc: +11.50Db During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.50dB During T5: Noc: -98dBm /15kHz Ês1 / Noc: +8.00 Ês2 / Noc: +11.50Db |  |
| 5.1.46 E-UTRAN TDD - TDD Intra frequency DAPS handover |  |  | Same as 5.1.45 |  |  | Same as 5.1.45 |  |  |  |  |  |  | Same as 5.1.45 |  |
| 5.1.47 E-UTRAN FDD - FDD Intra frequency conditional handover |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.5dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.50dB |  |
| 5.1.48 E-UTRAN TDD - TDD Intra frequency conditional handover |  |  | Same as 5.1.47 |  |  | Same as 5.1.47 |  |  |  |  |  |  | Same as 5.1.47 |  |
| 5.1.49 E-UTRAN FDD - FDD Inter frequency conditional handover |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7dB |  |
| 5.1.50 E-UTRAN TDD - TDD Inter frequency conditional handover |  |  | Same as 5.1.49 |  |  | Same as 5.1.49 |  |  |  |  |  |  | Same as 5.1.49 |  |
| 5.1.51 E-UTRAN FDD - TDD Inter frequency conditional handover |  |  | Same as 5.1.49 |  |  | Same as 5.1.49 |  |  |  |  |  |  | Same as 5.1.49 |  |
| 5.1.52 E-UTRAN TDD - FDD Inter frequency conditional handover |  |  | Same as 5.1.49 |  |  | Same as 5.1.49 |  |  |  |  |  |  | Same as 5.1.49 |  |
| 5.1.53 E-UTRAN TDD – TDD Intra-band Inter-frequency sync DAPS handover |  |  | Same as 5.1.41 |  |  | Same as 5.1.41 |  |  |  |  |  |  | Same as 5.1.41 |  |
| 5.1.54 E-UTRAN TDD – TDD Inter-band Inter-frequency sync DAPS handover |  |  | Same as 5.1.41 |  |  | Same as 5.1.41 |  |  |  |  |  |  | Same as 5.1.41 |  |
| 5.1.55 E-UTRAN FDD - TDD inter-band inter-frequency synchronous DAPS handover |  |  | Same as 5.1.41 |  |  | Same as 5.1.41 |  |  |  |  |  |  | Same as 5.1.41 |  |
| 5.1.56 E-UTRAN TDD - FDD inter-band inter-frequency synchronous DAPS handover |  |  | Same as 5.1.41 |  |  | Same as 5.1.41 |  |  |  |  |  |  | Same as 5.1.41 |  |
| 5.1.57 E-UTRAN FDD – TDD Inter-band Inter-frequency async DAPS handover |  |  | Same as 5.1.41 |  |  | Same as 5.1.41 |  |  |  |  |  |  | Same as 5.1.41 |  |
| 5.1.58 E-UTRAN TDD – FDD Inter-band Inter-frequency async DAPS handover |  |  | Same as 5.1.41 |  |  | Same as 5.1.41 |  |  |  |  |  |  | Same as 5.1.41 |  |
| 5.2.1 E-UTRAN FDD - UTRAN FDD handover |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 0.00dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 0.00dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc:-1.80dB During T3: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 0.00dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc:-1.80dB |  |  | During T1: 0dB -1.10dB 0dB - During T2: 0dB -1.10dB 0dB 0dB During T3: 0dB -1.10dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: -1.10dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: -1.10dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc:-1.80dB During T3: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: -1.10dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc:-1.80dB |  |
| 5.2.2 E-UTRAN TDD - UTRAN FDD handover |  |  | Same as 5.2.1 |  |  | Same as 5.2.1 |  |  |  |  |  |  | Same as 5.2.1 |  |
| 5.2.3 E-UTRAN FDD - GSM handover |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -85dBm During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -75dBm During T3: E-UTRA Cell 1 Noc: -98dBm /15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -75dBm |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRAN Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -85dBm During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -75dBm During T3: E-UTRA Cell 1 Noc: -98dBm /15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -75dBm |  |
| 5.2.4 E-UTRA TDD - UTRA TDD handover |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +13.00dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Îor / Ioc: -3.00dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: -3.00dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Îor / Ioc: 11.00dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T3: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: -3.00dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Îor / Ioc: 11.00dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |  | During T1: -1.05dB 2.1dB -0.8dB 0dB 0dB 0dB During T2: -1.05dB 0dB -0.8dB 1.6dB 0dB 0dB During T3: -1.05dB 0dB -0.8dB +1.6dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc: -99.05dBm/15kHz Ês / Noc: +15.1dB Ioc: -80.8dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: Noc: -99.05dBm/15kHz Ês / Noc: -3.0dB Ioc: -80.8dBm/1.28MHz Îor / Ioc: 12.6dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T3: Noc: -99.05dBm/15kHz Ês / Noc: -3.0dB Ioc: -80.8dBm/1.28MHz Îor / Ioc: 12.6dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |
| 5.2.5 E-UTRA FDD - UTRA TDD handover |  |  | Same as 5.2.4 |  |  | Same as 5.2.4 |  |  |  |  |  |  | Same as 5.2.4 |  |
| 5.2.6 E-UTRA TDD - GSM handover |  |  | Same as 5.2.3 |  |  | Same as 5.2.3 |  |  |  |  |  |  | Same as 5.2.3 |  |
| 5.2.7 E-UTRAN FDD - UTRAN FDD handover: unknown target cell |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 0dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 0dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -1.8 dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 0dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 0dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -1.8dB |  |
| 5.2.8 E-UTRAN FDD - GSM handover: unknown target cell |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -75 dBm |  |  | During T1: 0dB 0dB During T2: 0dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -75 dBm |  |
| 5.2.9 E-UTRAN TDD - GSM handover: unknown target cell |  |  | Same as 5.2.8 |  |  | Same as 5.2.8 |  |  |  |  |  |  | Same as 5.2.8 |  |
| 5.2.10 E-UTRAN TDD - UTRAN TDD HO test: unknown target cell |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 3dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 3dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: 13 dB PCCPCH_Ec/Ior: -3.00dB DwPCH_Ec/Ior: 0dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 3dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 3dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: 13 dB PCCPCH_Ec/Ior: -3.00dB DwPCH_Ec/Ior: 0dB |  |
| 5.2.11 E-UTRAN FDD - UTRAN FDD Handover for 5MHz Bandwidth |  |  | Same as 5.2.1 |  |  | Same as 5.2.1 |  |  |  |  |  |  | Same as 5.2.1 |  |
| 5.3.1 RRC CONNECTED / Handover from E-UTRAN to non-3GPP RATs / E-UTRAN FDD - HRPD handover |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 0dB HRPD Cell 2 Ioc: -55.00dBm/1.2288MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 0dB HRPD Cell 2 Ioc: -55.00dBm/1.2288MHz Ior / Ioc: 0 dB During T3: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 0dB HRPD Cell 2 Ioc: -55.00dBm/1.2288MHz Ior / Ioc: 0 dB |  |  | During T1: 0dB -0.8dB 0dB During T2: 0dB -0.8dB 0dB 0dB During T3: 0dB -0.8dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês / Noc: -0.8dB Ioc: -55.00dBm/1.2288MHz Ior / Ioc: -infinity During T2: Noc: -98dBm/15kHz Ês / Noc: -0.8dB Ioc: -55.00dBm/1.2288MHz Ior / Ioc: 0 dB During T3: Noc: -98dBm/15kHz Ês / Noc: -0.8dB Ioc: -55.00dBm/1.2288MHz Ior / Ioc: 0 dB |  |
| 5.3.5 E-UTRAN TDD - HRPD handover |  |  | Same as 5.3.1 |  |  | Same as 5.3.1 |  |  |  |  |  |  | Same as 5.3.1 |  |
| 6.1.1 E-UTRAN FDD Intra-frequency RRC Re-establishment |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +7.00dB Ês2 / Noc: +4.00dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +7.00dB Ês2 / Noc: +4.00dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4.00dB |  |
| 6.1.1_2 E-UTRAN FDD Intra-frequency RRC Re-establishment for UE Category 1bis |  |  | Same as 6.1.1 |  |  | Same as 6.1.1 |  |  |  |  |  |  | Same as 6.1.1 |  |
| 6.1.2 E-UTRAN FDD Inter-frequency RRC Re-establishment |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -infinity Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: -infinity Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -infinity Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: -infinity Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB |  |
| 6.1.2_2 E-UTRAN FDD Inter-frequency RRC Re-establishment for UE Category 1bis |  |  | Same as 6.1.2 |  |  | Same as 6.1.2 |  |  |  |  |  |  | Same as 6.1.2 |  |
| 6.1.3 E-UTRAN TDD Intra-frequency RRC Re-establishment |  |  | Same as 6.1.1 |  |  | Same as 6.1.1 |  |  |  |  |  |  | Same as 6.1.1 |  |
| 6.1.3_2 E-UTRAN TDD Intra-frequency RRC Re-establishment for UE Category 1bis |  |  | Same as 6.1.1 |  |  | Same as 6.1.1 |  |  |  |  |  |  | Same as 6.1.1 |  |
| 6.1.4 E-UTRAN TDD Inter-frequency RRC Re-establishment |  |  | Same as 6.1.3 |  |  | Same as 6.1.3 |  |  |  |  |  |  | Same as 6.1.3 |  |
| 6.1.4_2 E-UTRAN TDD Inter-frequency RRC Re-establishment for UE Category 1bis |  |  | Same as 6.1.2 |  |  | Same as 6.1.2 |  |  |  |  |  |  | Same as 6.1.2 |  |
| 6.1.5 E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz Bandwidth |  |  | Same as 6.1.1 |  |  | Same as 6.1.1 |  |  |  |  |  |  | Same as 6.1.1 |  |
| 6.1.6 E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0 |  |  | Same as 6.1.1 |  |  | Same as 6.1.1 |  |  |  |  |  |  | Same as 6.1.1 |  |
| 6.1.7 E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0 |  |  | Same as 6.1.1 |  |  | Same as 6.1.1 |  |  |  |  |  |  | Same as 6.1.1 |  |
| 6.1.8 E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0 |  |  | Same as 6.1.1 |  |  | Same as 6.1.1 |  |  |  |  |  |  | Same as 6.1.1 |  |
| 6.1.9 E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA |  |  | Same as 6.1.1 |  |  | Same as 6.1.1 |  |  |  |  |  |  | Same as 6.1.1 |  |
| 6.1.10 E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA |  |  | Same as 6.1.1 |  |  | Same as 6.1.1 |  |  |  |  |  |  | Same as 6.1.1 |  |
| 6.1.11 E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA |  |  | Same as 6.1.1 |  |  | Same as 6.1.1 |  |  |  |  |  |  | Same as 6.1.1 |  |
| 6.1.12 E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +7.00dB Ês2 / Noc: +4.00dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: -15.00dB Ês2 / Noc: -12.00dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: -12.00dB During T4: Noc: -98dBm /15kHz Ês1 / Noc: -infinity Ês2 / Noc: -12.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB +0.6dB 0dB During T3: 0dB 0dB 0dB During T4: 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +7.00dB Ês2 / Noc: +4.00dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: -14.40dB Ês2 / Noc: -12.00dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: -infinity Ês2 / Noc: -12.00dB |  |
| 6.1.13 E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB |  |  | Same as 6.1.12 |  |  | Same as 6.1.12 |  |  |  |  |  |  | Same as 6.1.12 |  |
| 6.1.14 E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB |  |  | Same as 6.1.12 |  |  | Same as 6.1.12 |  |  |  |  |  |  | Same as 6.1.12 |  |
| 6.1.15 HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: -12.6dB Êsn1 / Noc: +7.0dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -12.6dB Êsn1 / Noc: -infinity Êsn2 / Noc: -12.6dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: -12.6dB Êsn1 / Noc: -infinity Êsn2 / Noc: -12.6dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: -12.6dB Êsn1 / Noc: +7.0dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -12.6dB Êsn1 / Noc: -infinity Êsn2 / Noc: -12.6dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: -12.6dB Êsn1 / Noc: -infinity Êsn2 / Noc: -12.6dB |  |
| 6.1.16 HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.0dB Êsn1 / Noc: +7.0dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.0dB Êsn1 / Noc: -infinity Êsn2 / Noc: +4.0dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +4.0dB Êsn1 / Noc: -infinity Êsn2 / Noc: +4.0dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.0dB Êsn1 / Noc: +7.0dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.0dB Êsn1 / Noc: -infinity Êsn2 / Noc: +4.0dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +4.0dB Êsn1 / Noc: -infinity Êsn2 / Noc: +4.0dB |  |
| 6.1.18 E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for CE UE in CEModeA |  |  | Same as 6.1.2 |  |  | Same as 6.1.2 |  |  |  |  |  |  | Same as 6.1.2s |  |
| 6.1.23 E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage |  |  | Same as 6.1.16 |  |  | Same as 6.1.16 |  |  |  |  |  |  | Same as 6.1.16 |  |
| 6.1.24 E-UTRAN TDD - TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage |  |  | Same as 6.1.15 |  |  | Same as 6.1.15 |  |  |  |  |  |  | Same as 6.1.15 |  |
| 6.2.1 E-UTRAN FDD - Contention Based Random Access Test |  |  | Test 1 and Test 2 Absolute uplink power: Normal conditions ±9dB Extreme conditions ±12dB Relative uplink power step: Normal conditions ±3dB Extreme conditions ±5dB Uplink timing Te: ±12Ts |  |  | 1.5dB 1.5dB 0.7dB 0.7dB 3Ts |  |  |  |  |  |  | Test 1 and Test 2 Absolute uplink power: Normal conditions ±10.5dB Extreme conditions ±13.5dB Relative uplink power step: Normal conditions ±3.7dB Extreme conditions ±5.7dB Uplink timing Te: ±15Ts |  |
| 6.2.1_2 E-UTRAN FDD - Contention Based Random Access Test |  |  | Same as 6.2.1 |  |  | Same as 6.2.1 |  |  |  |  |  |  | Same as 6.2.1 |  |
| 6.2.2 E-UTRAN FDD - Non Contention Based Random Access Test |  |  | Same as 6.2.1 |  |  | Same as 6.2.1 |  |  |  |  |  |  | Same as 6.2.1 |  |
| 6.2.2_2 E-UTRAN FDD - Non Contention Based Random Access Test |  |  | Same as 6.2.1 |  |  | Same as 6.2.1 |  |  |  |  |  |  | Same as 6.2.1 |  |
| 6.2.3 E-UTRAN TDD - Contention Based Random Access Test |  |  | Same as 6.2.1 |  |  | Same as 6.2.1 |  |  |  |  |  |  | Same as 6.2.1 |  |
| 6.2.3_2 E-UTRAN TDD - Contention Based Random Access Test |  |  | Same as 6.2.1 |  |  | Same as 6.2.1 |  |  |  |  |  |  | Same as 6.2.1 |  |
| 6.2.4 E-UTRAN TDD - Non Contention Based Random Access Test |  |  | Same as 6.2.1 |  |  | Same as 6.2.1 |  |  |  |  |  |  | Same as 6.2.1 |  |
| 6.2.4_2 E-UTRAN TDD - Non Contention Based Random Access Test |  |  | Same as 6.2.1 |  |  | Same as 6.2.1 |  |  |  |  |  |  | Same as 6.2.1 |  |
| 6.2.5 E-UTRAN FDD - Contention Based Random Access Test for 5MHz bandwidth |  |  | Same as 6.2.1 |  |  | Same as 6.2.1 |  |  |  |  |  |  | Same as 6.2.1 |  |
| 6.2.6 E-UTRAN FDD - Non-contention Based Random Access Test for 5MHz bandwidth |  |  | Same as 6.2.1 |  |  | Same as 6.2.1 |  |  |  |  |  |  | Same as 6.2.1 |  |
| 6.2.7 E-UTRAN FDD - Non-Contention Based Random Access Test for SCell in sTAG |  |  | Test 1, Test 2 and Test 3 Absolute uplink power: Normal conditions ±9dB Extreme conditions ±12dB Relative uplink power step: Normal conditions ±3dB Extreme conditions ±5dB Uplink timing Te: ±12Ts |  |  | 1.5dB 1.5dB 0.7dB 0.7dB 3Ts |  |  |  |  |  |  | Test 1, Test 2 and Test 3 Absolute uplink power: Normal conditions ±10.5dB Extreme conditions ±13.5dB Relative uplink power step: Normal conditions ±3.7dB Extreme conditions ±5.7dB Uplink timing Te: ±15Ts |  |
| 6.2.8 E-UTRAN TDD - Non-Contention Based Random Access Test for SCell in sTAG |  |  | Same as 6.2.7 |  |  | Same as 6.2.7 |  |  |  |  |  |  | Same as 6.2.7 |  |
| 6.2.9 3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells |  |  | Same as 6.2.7 |  |  | Same as 6.2.7 |  |  |  |  |  |  | Same as 6.2.7 |  |
| 6.2.10 E-UTRAN FDD - Contention based random access test for Cat-M1 UEs in normal coverage |  |  | Test 1 and Test 2 Absolute uplink power: Normal conditions ±9dB Extreme conditions ±12dB Relative uplink power step: Normal conditions ±3dB Extreme conditions ±5dB Timing Te: ±24Ts rsrp-ThresholdsPrach {24, 27, 33} Noc: -103dBm/15kHz Ês / Noc: +3dB |  |  | 1.5dB 1.5dB 0.7dB 0.7dB 3Ts {0, 0, -2} for normal {-3, -3, -5} for extreme -0.4dB -0.3dB |  |  |  |  |  |  | Test 1 and Test 2 Absolute uplink power: Normal conditions ±10.5dB Extreme conditions ±13.5dB Relative uplink power step: Normal conditions ±3.7dB Extreme conditions ±5.7dB Uplink timing Te: ±27Ts rsrp-ThresholdsPrach {24, 27, 31} for normal conditions and {21, 24, 28} for extreme conditions Noc: -103.4dBm/15kHz Ês / Noc: +2.7dB |  |
| 6.2.11 E-UTRAN HD-FDD - Contention based random access test for Cat-M1 UEs in normal coverage |  |  | Same as 6.2.10 |  |  | Same as 6.2.10 |  |  |  |  |  |  | Same as 6.2.10 |  |
| 6.2.12 E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage |  |  | Same as 6.2.1 |  |  | Same as 6.2.10 |  |  |  |  |  |  | Same as 6.2.10 |  |
| 6.2.13 E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage |  |  | Test 1 and Test 2 Absolute uplink power: Normal conditions ±9dB Extreme conditions ±12dB Relative uplink power step: Normal conditions ±3dB Extreme conditions ±5dB Timing Te: ±48Ts rsrp-ThresholdsPrach {23, 37, 47} Noc: -98dBm/15kHz Ês / Noc: -12dB |  |  | 1.5dB 1.5dB 0.7dB 0.7dB 3Ts {-2, 1, 0} for normal {-5, 4, 3} for extreme -1.1dB 0.6dB |  |  |  |  |  |  | Test 1 and Test 2 Absolute uplink power: Normal conditions ±10.5dB Extreme conditions ±13.5dB Relative uplink power step: Normal conditions ±3.7dB Extreme conditions ±5.7dB Uplink timing Te: ±51Ts rsrp-ThresholdsPrach {21, 38, 47} for normal conditions and {18, 41, 50} for extreme conditions Noc: -99.1dBm/15kHz Ês / Noc: -11.4dB |  |
| 6.2.14 E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage |  |  | Same as 6.2.13 |  |  | Same as 6.2.13 |  |  |  |  |  |  | Same as 6.2.13 |  |
| 6.2.15 E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage |  |  | Same as 6.2.13 |  |  | Same as 6.2.13 |  |  |  |  |  |  | Same as 6.2.13 |  |
| 6.2.16 Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage |  |  | Absolute uplink power: Normal conditions ±9dB Extreme conditions ±12dB Relative uplink power step: Normal conditions ±2dB Extreme conditions ±4dB Uplink timing Te: ±80Ts rsrp-ThresholdsPrach {40, 55} Ês / Noc: 3dB |  |  | 1.4dB 1.4dB 0.7dB 0.7dB 3Ts {0, -1} for normal {0, -4} for extreme 0.1dB |  |  |  |  |  |  | Absolute uplink power: Normal conditions ±10.4dB Extreme conditions ±13.4dB Relative uplink power step: Normal conditions ±2.7dB Extreme conditions ±4.7dB Uplink timing Te: ±83Ts rsrp-ThresholdsPrach {40, 54} for normal conditions and {40, 51} for extreme conditions Ês / Noc: 3.1dB |  |
| 6.2.17 Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage |  |  | Absolute uplink power: Normal conditions ±13.3dB Extreme conditions ±16.3dB Relative uplink power step: Normal conditions ±2dB Extreme conditions ±4dB Uplink timing Te: ±80Ts rsrp-ThresholdsPrach {35, 56} |  |  | 1.0dB 1.0dB 0.7dB 0.7dB 3Ts {-1, 1} for normal {-4, 4} for extreme |  |  |  |  |  |  | Absolute uplink power: Normal conditions ±14.3dB Extreme conditions ±17.3dB Relative uplink power step: Normal conditions ±2.7dB Extreme conditions ±4.7dB Uplink timing Te: ±83Ts rsrp-ThresholdsPrach {34, 57} for normal conditions and {31, 60} for extreme conditions |  |
| 6.2.18 Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage |  |  | Same as 6.2.17 |  |  | Same as 6.2.17 |  |  |  |  |  |  | Same as 6.2.17 |  |
| 6.2.19 TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage |  |  | Same as 6.2.16 |  |  | Same as 6.2.16 |  |  |  |  |  |  | Same as 6.2.16 |  |
| 6.2.20 TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in enhanced coverage |  |  | Same as 6.2.17 |  |  | Same as 6.2.17 |  |  |  |  |  |  | Same as 6.2.17 |  |
| 6.2.21 TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage |  |  | Same as 6.2.17 |  |  | Same as 6.2.17 |  |  |  |  |  |  | Same as 6.2.17 |  |
| 6.3.1 Redirection from E-UTRAN FDD to UTRAN FDD |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: 0.02 dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0.4dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: 0.42dB |  |
| 6.3.1_2 Redirection from E-UTRAN FDD to UTRAN FDD |  |  | Same as 6.3.1 |  |  | Same as 6.3.1 |  |  |  |  |  |  | Same as 6.3.1 |  |
| 6.3.2 Redirection from E-UTRAN TDD to UTRAN FDD |  |  | Same as 6.3.1 |  |  | Same as 6.3.1 |  |  |  |  |  |  | Same as 6.3.1 |  |
| 6.3.2_2 Redirection from E-UTRAN TDD to UTRAN FDD |  |  | Same as 6.3.1 |  |  | Same as 6.3.1 |  |  |  |  |  |  | Same as 6.3.1 |  |
| 6.3.3 Redirection from E-UTRAN FDD to GERAN when System Information is provided |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -75 dBm |  |  | During T1: 0dB 0dB During T2: 0dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -75 dBm |  |
| 6.3.3_2 Redirection from E-UTRAN FDD to GERAN when System Information is provided |  |  | Same as 6.3.3 |  |  | Same as 6.3.3 |  |  |  |  |  |  | Same as 6.3.3 |  |
| 6.3.4 Redirection from E-UTRAN TDD to GERAN when System Information is provided |  |  | Same as 6.3.3 |  |  | Same as 6.3.3 |  |  |  |  |  |  | Same as 6.3.3 |  |
| 6.3.4_2 Redirection from E-UTRAN TDD to GERAN when System Information is provided |  |  | Same as 6.3.3 |  |  | Same as 6.3.3 |  |  |  |  |  |  | Same as 6.3.3 |  |
| 6.3.5 E-UTRA TDD RRC connection release redirection to UTRA TDD |  |  | During T1: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -infinity PCCPCH_Ec/Ior: -4.77dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: +4.0dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: +8.0dB PCCPCH_Ec/Ior: -4.77dB DwPCH_Ec/Ior: 0dB |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -infinity PCCPCH_Ec/Ior: -4.77dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: +4.0dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: +8.0dB PCCPCH_Ec/Ior: -4.77dB DwPCH_Ec/Ior: 0dB |  |
| 6.3.5_2 E-UTRA TDD RRC connection release redirection to UTRA TDD |  |  | Same as 6.3.5 |  |  | Same as 6.3.5 |  |  |  |  |  |  | Same as 6.3.5 |  |
| 6.3.6 E-UTRA FDD RRC connection release redirection to UTRA TDD |  |  | Same as 6.3.5 |  |  | Same as 6.3.5 |  |  |  |  |  |  | Same as 6.3.5 |  |
| 6.3.6_2 E-UTRA FDD RRC connection release redirection to UTRA TDD |  |  | Same as 6.3.5 |  |  | Same as 6.3.5 |  |  |  |  |  |  | Same as 6.3.5 |  |
| 6.3.7 E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided |  |  | Same as 6.3.5 |  |  | Same as 6.3.5 |  |  |  |  |  |  | Same as 6.3.5 |  |
| 6.3.7_2 E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided |  |  | Same as 6.3.5 |  |  | Same as 6.3.5 |  |  |  |  |  |  | Same as 6.3.5 |  |
| 6.3.8 E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided |  |  | Same as 6.3.5 |  |  | Same as 6.3.5 |  |  |  |  |  |  | Same as 6.3.5 |  |
| 6.3.8_2 E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided |  |  | Same as 6.3.5 |  |  | Same as 6.3.5 |  |  |  |  |  |  | Same as 6.3.5 |  |
| 6.3.9 Redirection from E-UTRAN FDD to UTRAN FDD without System Information |  |  | Same as 6.3.1 |  |  | Same as 6.3.1 |  |  |  |  |  |  | Same as 6.3.1 |  |
| 6.3.9_2 Redirection from E-UTRAN FDD to UTRAN FDD without System Information |  |  | Same as 6.3.1 |  |  | Same as 6.3.1 |  |  |  |  |  |  | Same as 6.3.1 |  |
| 6.3.10 Redirection from E-UTRAN FDD to GERAN when System Information is not provided |  |  | Same as 6.3.3 |  |  | Same as 6.3.3 |  |  |  |  |  |  | Same as 6.3.3 |  |
| 6.3.10_2 Redirection from E-UTRAN FDD to GERAN when System Information is not provided |  |  | Same as 6.3.3 |  |  | Same as 6.3.3 |  |  |  |  |  |  | Same as 6.3.3 |  |
| 6.3.11 Redirection from E-UTRAN TDD to GERAN when System Information is not provided |  |  | Same as 6.3.3 |  |  | Same as 6.3.3 |  |  |  |  |  |  | Same as 6.3.3 |  |
| 6.3.11_2 Redirection from E-UTRAN TDD to GERAN when System Information is not provided |  |  | Same as 6.3.3 |  |  | Same as 6.3.3 |  |  |  |  |  |  | Same as 6.3.3 |  |
| 6.3.12 E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided |  |  | Same as 6.3.1 |  |  | Same as 6.3.1 |  |  |  |  |  |  | Same as 6.3.1 |  |
| 6.3.12_2 E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided |  |  | Same as 6.3.1 |  |  | Same as 6.3.1 |  |  |  |  |  |  | Same as 6.3.1 |  |
| 7.1.1 E-UTRAN FDD - UE Transmit Timing Accuracy |  |  | Test 1 (10MHz Ch BW): Uplink timing: ±12Ts Max step size Tq: 3.5Ts Min adjust rate: 7Ts Max adjust rate: 3.5Ts Ês / Noc: +3.00dB Test 2 (10MHz Ch BW): Uplink timing: ±12Ts Ês / Noc: +3.00dB Test 3: (1.4MHz Ch BW) Uplink timing: ±24Ts Max step size Tq: 17.5Ts Max adjust rate: 17.5Ts Ês / Noc: +3.00dB |  |  | ±3Ts +0.5Ts -3.6Ts +1.1Ts +0.3dB ±3Ts +0.3dB ±3Ts +0.5Ts +1.1Ts +0.3dB |  |  |  |  |  |  | Test 1 (10MHz Ch BW): Uplink timing: ±15Ts Max step size Tq: 4.0T Min adjust rate: 3.4Ts Max adjust rate: 4.6Ts Ês / Noc: +3.30dB Test 2 (10MHz Ch BW): Uplink timing: ±15Ts Ês / Noc: +3.30dB Test 3: (1.4MHz Ch BW) Uplink timing: ±27Ts Max step size Tq: 18Ts Max adjust rate: 18.6Ts Ês / Noc: +3.30dB |  |
| 7.1.1_1 E-UTRAN FDD - UE Transmit Timing Accuracy (Non DRx UE) |  |  | Test 1 (10MHz Ch BW): Uplink timing: ±12Ts Max step size Tq: 3.5Ts Min adjust rate: 7Ts Max adjust rate: 3.5Ts Ês / Noc: +3.00dB Test 2 not applicable Test 3: (1.4MHz Ch BW) Uplink timing: ±24Ts Max step size Tq: 17.5Ts Max adjust rate: 17.5Ts Ês / Noc: +3.00dB |  |  | ±3Ts +0.5Ts -3.6Ts +1.1Ts +0.3dB ±3Ts +0.5Ts +1.1Ts +0.3dB |  |  |  |  |  |  | Test 1 (10MHz Ch BW): Uplink timing: ±15Ts Max step size Tq: 4Ts Min adjust rate: 3.4Ts Max adjust rate: 4.6Ts Ês / Noc: +3.30dB Test 2 not applicable Test 3: (1.4MHz Ch BW) Uplink timing: ±27Ts Max step size Tq: 18Ts Max adjust rate: 18.6Ts Ês / Noc: +3.30dB |  |
| 7.1.1_2 E-UTRAN FDD - UE Transmit Timing Accuracy for UE category 1bis |  |  | Same as 7.1.1 |  |  | Same as 7.1.1 |  |  |  |  |  |  | Same as 7.1.1 |  |
| 7.1.2 E-UTRAN TDD - UE Transmit Timing Accuracy |  |  | Test 1 (10MHz Ch BW): Uplink timing: (624 ±12) x Ts Max step size Tq: 3.5Ts Min adjust rate: 7Ts Max adjust rate: 3.5Ts Ês / Noc: +3.00dB Test 2 (10MHz Ch BW): Uplink timing: (624 ±12) x Ts Ês / Noc: +3.00dB Test 3: (1.4MHz Ch BW) Uplink timing: (624 ±24) x Ts Max step size Tq: 17.5Ts Max adjust rate: 17.5Ts Ês / Noc: +3.00dB |  |  | ±3Ts +0.5Ts -3.6Ts +1.1Ts +0.3dB ±3Ts +0.3dB ±3Ts +0.5Ts +1.1Ts +0.3dB |  |  |  |  |  |  | Test 1 (10MHz Ch BW): Uplink timing: (624 ±15) x Ts Max step size Tq: 4Ts Min adjust rate: 3.4Ts Max adjust rate: 4.6Ts Ês / Noc: +3.30dB Test 2 (10MHz Ch BW): Uplink timing: (624 ±15) x Ts Ês / Noc: +3.30dB Test 3: (1.4MHz Ch BW) Uplink timing: (624 ±27) x Ts Max step size Tq: 18Ts Max adjust rate: 18.6Ts Ês / Noc: +3.30dB |  |
| 7.1.2_1 E-UTRAN TDD - UE Transmit Timing Accuracy (Non DRx UE) |  |  | Test 1 (10MHz Ch BW): Uplink timing: (624 ±12) x Ts Max step size Tq: 3.5Ts Min adjust rate: 7Ts Max adjust rate: 3.5Ts Ês / Noc: +3.00dB Test 2 not applicable Test 3: (1.4MHz Ch BW) Uplink timing: (624 ±24) x Ts Max step size Tq: 17.5Ts Max adjust rate: 17.5Ts Ês / Noc: +3.00dB |  |  | ±3Ts +0.5Ts -3.6Ts +1.1Ts +0.3dB ±3Ts +0.5Ts +1.1Ts +0.3dB |  |  |  |  |  |  | Test 1 (10MHz Ch BW): Uplink timing: (624 ±15) x Ts Max step size Tq: 4Ts Min adjust rate: 3.4Ts Max adjust rate: 4.6Ts Ês / Noc: +3.30dB Test 2 not applicable Test 3: (1.4MHz Ch BW) Uplink timing: (624 ±27) x Ts Max step size Tq: 18Ts Max adjust rate: 18.6Ts Ês / Noc: +3.30dB |  |
| 7.1.2_2 E-UTRAN TDD - UE Transmit Timing Accuracy for UE category 1bis |  |  | Same as 7.1.2 |  |  | Same as 7.1.2 |  |  |  |  |  |  | Same as 7.1.2 |  |
| 7.1.3 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for SCell |  |  | Test 1: Uplink timing: ±12Ts Max step size Tq: 3.5Ts Min adjust rate: 7Ts Max adjust rate: 3.5Ts Ês1 / Noc1: +3.00dB Ês2 / Noc2: +3.00dB Test 2: Uplink timing: ±12Ts Ês1 / Noc1: +3.00dB Ês2 / Noc2: +3.00dB |  |  | ±3Ts +0.5Ts -3.6Ts +1.1Ts +0.3dB +0.3dB ±3Ts +0.3dB +0.3dB |  |  |  |  |  |  | Test 1: Uplink timing: ±15Ts Max step size Tq: 4Ts Min adjust rate: 3.4Ts Max adjust rate: 4.6Ts Ês1 / Noc1: +3.30dB Ês2 / Noc2: +3.30dB Test 2: Uplink timing: ±15Ts Ês1 / Noc1: +3.30dB Ês2 / Noc2: +3.30dB |  |
| 7.1.3_1 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for SCell (Release 12 and forward) |  |  | Test 1: Uplink timing: ±12Ts Max step size Tq: 3.5Ts Min adjust rate: 7Ts Max adjust rate: 3.5Ts Ês1 / Noc1: +3.00dB Ês2 / Noc2: +3.00dB Test 2 and Test 3: Uplink timing: ±12Ts Ês1 / Noc1: +3.00dB Ês2 / Noc2: +3.00dB |  |  | ±3Ts +0.5Ts -3.6Ts +1.1Ts +0.3dB +0.3dB ±3Ts +0.3dB +0.3dB |  |  |  |  |  |  | Test 1: Uplink timing: ±15Ts Max step size Tq: 4Ts Min adjust rate: 3.4Ts Max adjust rate: 4.6Ts Ês1 / Noc1: +3.30dB Ês2 / Noc2: +3.30dB Test 2 and Test 3: Uplink timing: ±15Ts Ês1 / Noc1: +3.30dB Ês2 / Noc2: +3.30dB |  |
| 7.1.4 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell |  |  | Test 1: Uplink timing: (624 ±12) x Ts Max step size Tq: 3.5Ts Min adjust rate: 7Ts Max adjust rate: 3.5Ts Ês1 / Noc1: +3.00dB Ês2 / Noc2: +3.00dB Test 2: Uplink timing: (624 ±12) x Ts Ês1 / Noc1: +3.00dB Ês2 / Noc2: +3.00dB |  |  | ±3Ts +0.5Ts -3.6Ts +1.1Ts +0.3dB +0.3dB ±3Ts +0.3dB +0.3dB |  |  |  |  |  |  | Test 1: Uplink timing: (624 ±15) x Ts Max step size Tq: 4Ts Min adjust rate: 3.4Ts Max adjust rate: 4.6Ts Ês1 / Noc1: +3.30dB Ês2 / Noc2: +3.30dB Test 2: Uplink timing: (624 ±15) x Ts Ês1 / Noc1: +3.30dB Ês2 / Noc2: +3.30dB |  |
| 7.1.4_1 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell (Release 12 and forward) |  |  | Test 1: Uplink timing: (624 ±12) x Ts Max step size Tq: 3.5Ts Min adjust rate: 7Ts Max adjust rate: 3.5Ts Ês1 / Noc1: +3.00dB Ês2 / Noc2: +3.00dB Test 2 and Test: Uplink timing: (624 ±12) x Ts Ês1 / Noc1: +3.00dB Ês2 / Noc2: +3.00dB |  |  | ±3Ts +0.5Ts -3.6Ts +1.1Ts +0.3dB +0.3dB ±3Ts +0.3dB +0.3dB |  |  |  |  |  |  | Test 1: Uplink timing: (624 ±15) x Ts Max step size Tq: 4Ts Min adjust rate: 3.4Ts Max adjust rate: 4.6Ts Ês1 / Noc1: +3.30dB Ês2 / Noc2: +3.00dB Test 2 and Test 3: Uplink timing: (624 ±15) x Ts Ês1 / Noc1: +3.30dB Ês2 / Noc2: +3.30dB |  |
| 7.1.4A E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz +10 MHz bandwidth |  |  | Same as 7.1.4 |  |  | Same as 7.1.4 |  |  |  |  |  |  | Same as 7.1.4 |  |
| 7.1.5 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for 5MHz Bandwidth |  |  | Test 1 (5MHz Ch BW): Uplink timing: ±12Ts Max step size Tq: 5.5Ts Min adjust rate: 11Ts in 1.57s Max adjust rate: 5.5Ts Ês / Noc: +3.00dB |  |  | ±3Ts +0.5Ts -5.3Ts +1.1Ts +0.3dB |  |  |  |  |  |  | Test 1 (5MHz Ch BW): Uplink timing: ±15Ts Max step size Tq: 6.0Ts Min adjust rate: 5.7Ts in 1.57s Max adjust rate: 6.6Ts Ês / Noc: +3.30dB |  |
| 7.1.6 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for SCell in sTAG |  |  | Test 1: Uplink timing: ±12Ts Max step size Tq: 3.5Ts Min adjust rate: 7Ts Max adjust rate: 3.5Ts Ês1 / Noc1: +3.00dB Ês2 / Noc2: +3.00dB Test 2: Uplink timing: ±12Ts Ês1 / Noc1: +3.00dB Ês2 / Noc2: +3.00dB |  |  | ±3Ts +0.5Ts -3.6Ts +1.1Ts +0.3dB +0.3dB ±3Ts +0.3dB +0.3dB |  |  |  |  |  |  | Test 1: Uplink timing: ±15Ts Max step size Tq: 4.0Ts Min adjust rate: 3.4Ts Max adjust rate: 4.6Ts Ês1 / Noc1: +3.30dB Ês2 / Noc2: +3.30dB Test 2: Uplink timing: ±15Ts Ês1 / Noc1: +3.30dB Ês2 / Noc2: +3.30dB |  |
| 7.1.7 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG |  |  | Test 1: Uplink timing: (624 ±12) x Ts Max step size Tq: 3.5Ts Min adjust rate: 7Ts Max adjust rate: 3.5Ts Ês1 / Noc1: +3.00dB Ês2 / Noc2: +3.00dB Test 2: Uplink timing: (624 ±12) x Ts Ês1 / Noc1: +3.00dB Ês2 / Noc2: +3.00dB |  |  | ±3Ts +0.5Ts -3.6Ts +1.1Ts +0.3dB +0.3dB ±3Ts +0.3dB +0.3dB |  |  |  |  |  |  | Test 1: Uplink timing: (624 ±15) x Ts Max step size Tq: 4.0Ts Min adjust rate: 3.4Ts Max adjust rate: 4.6Ts Ês1 / Noc1: +3.30dB Ês2 / Noc2: +3.30dB Test 2: Uplink timing: (624 ±15) x Ts Ês1 / Noc1: +3.30dB Ês2 / Noc2: +3.30dB |  |
| 7.1.7A E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz bandwidth |  |  | Sams as 7.1.7 |  |  | Sams as 7.1.7 |  |  |  |  |  |  | Sams as 7.1.7 |  |
| 7.1.7B E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz bandwidth |  |  | Sams as 7.1.7 |  |  | Sams as 7.1.7 |  |  |  |  |  |  | Sams as 7.1.7 |  |
| 7.1.10 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |  |  |  |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |
| 7.1.11 E-UTRAN HD-FDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |  |  |  |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |
| 7.1.12 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |  |  |  |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |
| 7.1.13 3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells |  |  | Test 1: Uplink timing: (624 ±12) x Ts Max step size Tq: 5.5Ts for 5MHz BW 3.5Ts for 10 MHz BW Min adjust rate: 7Ts Max adjust rate: 5.5Ts for 5MHz BW 3.5Ts for 10 MHz BW Ês1 / Noc1: +3.00dB Ês2 / Noc2: +3.00dB Ês3 / Noc3: +3.00dB Test 2: Uplink timing: (624 ±12) x Ts Ês1 / Noc1: +3.00dB Ês2 / Noc2: +3.00dB Ês3 / Noc3: +3.00dB |  |  | ±3Ts +0.5Ts +0.5Ts -3.6Ts +1.1Ts +1.1Ts +0.3dB +0.3dB +0.3dB ±3Ts +0.3dB +0.3dB +0.3dB |  |  |  |  |  |  | Test 1: Uplink timing: (624 ±15) x Ts Max step size Tq: 6.0Ts for 5MHz BW 4.0Ts for 10 MHz BW Min adjust rate: 3.4Ts Max adjust rate: 6.6Ts for 5MHz BW 4.6Ts for 10 MHz BW Ês1 / Noc1: +3.30dB Ês2 / Noc2: +3.30dB Ês3 / Noc3: +3.30dB Test 2: Uplink timing: (624 ±15) x Ts Ês1 / Noc1: +3.30dB Ês2 / Noc2: +3.30dB Ês3 / Noc3: +3.30dB |  |
| 7.1.14 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB |  |  | Uplink timing: ±48Ts Max step size Tq: 17.5Ts Max adjust rate: 17.5Ts Ês / Noc: -12.00dB |  |  | ±3Ts +0.5Ts +1.1Ts(f > 1GHz) +1.7Ts(f ≤ 1GHz) +0.3dB |  |  |  |  |  |  | Uplink timing: ±51Ts Max step size Tq: 18Ts Max adjust rate: 18.6Ts(f > 1GHz) Max adjust rate: 19.2Ts(f ≤ 1GHz) Ês / Noc: -11.70dB |  |
| 7.1.15 E-UTRAN HD-FDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB |  |  | Same as 7.1.14 |  |  | Same as 7.1.14 |  |  |  |  |  |  | Same as 7.1.14 |  |
| 7.1.16 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB |  |  | Same as 7.1.14 |  |  | Same as 7.1.14 |  |  |  |  |  |  | Same as 7.1.14 |  |
| 7.1.17 E-UTRAN HD-FDD - UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage |  |  | Test 1 Uplink timing: ±80Ts Max step size: 58.33Ts Min adjust rate: 7Ts Max adjust rate: 58.33Ts Ês / Noc: +4.00dB |  |  | ±3Ts +0.5Ts -3.6Ts for carrier frequency under 1 GHz -6.6Ts for carrier frequency over 1 GHz +1.1Ts for carrier frequency under 1 GHz +1.7 Ts for carrier frequency over 1 GHz +0.3dB |  |  |  |  |  |  | Test 1 Uplink timing: ±83Ts Max step size: 58.83T Min adjust rate: 3.4Ts for carrier frequency under 1 GHz 0.4Ts for carrier frequency over 1 GHz Max adjust rate: 59.43Ts for carrier frequency under 1 GHz 60.03Ts for carrier frequency over 1 GHz Ês / Noc: +4.30dB |  |
| 7.1.18 E-UTRAN HD-FDD - UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under enhanced coverage |  |  | Test 1&Test 2 Uplink timing: ±80Ts Test 1 Uplink timing: 0Ts (during ongoing repetition) Ês / Noc: +4.00dB |  |  | ±3Ts ±3Ts +0.3dB |  |  |  |  |  |  | Test 1&Test 2 Uplink timing: ±83Ts Test 1 Uplink timing: ±3Ts (during ongoing repetition) Ês / Noc: +4.30dB |  |
| 7.1.21 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |  |  |  |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |
| 7.1.22 E-UTRAN HD-FDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |  |  |  |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |
| 7.1.23 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |  |  |  |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |
| 7.1.24 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB |  |  | Uplink timing: ±40Ts Max step size Tq: 17.5Ts Max adjust rate: 17.5Ts Ês / Noc: -12.00dB |  |  | ±3Ts +0.5Ts +1.1Ts(f > 1GHz) +1.7Ts(f ≤ 1GHz) +0.3dB |  |  |  |  |  |  | Uplink timing: ±43Ts Max step size Tq: 18Ts Max adjust rate: 18.6Ts(f > 1GHz) Max adjust rate: 19.2Ts(f ≤ 1GHz) Ês / Noc: -11.70dB |  |
| 7.1.27 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage |  |  | Same as 7.1.17 |  |  | Same as 7.1.17 |  |  |  |  |  |  | Same as 7.1.17 |  |
| 7.1.28 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage |  |  | Same as 7.1.18 |  |  | Same as 7.1.18 |  |  |  |  |  |  | Same as 7.1.18 |  |
| 7.2.1 E-UTRAN FDD - UE Timing Advanced Adjustment Accuracy |  |  | Timing Advance Adjustment: ±4Ts |  |  | 0.5TS |  |  |  |  |  |  | Timing Advance Adjustment: ±4.5Ts |  |
| 7.2.1_2 E-UTRAN FDD - UE Timing Advance Adjustment Accuracy for UE category 1bis |  |  | Same as 7.2.1 |  |  | Same as 7.2.1 |  |  |  |  |  |  | Same as 7.2.1 |  |
| 7.2.2 E-UTRAN TDD - UE Timing Advance Adjustment Accuracy |  |  | Same as 7.2.1 |  |  | Same as 7.2.1 |  |  |  |  |  |  | Same as 7.2.1 |  |
| 7.2.2_2 E-UTRAN TDD - UE Timing Advance Adjustment Accuracy for UE category 1bis |  |  | Same as 7.2.1 |  |  | Same as 7.2.1 |  |  |  |  |  |  | Same as 7.2.1 |  |
| 7.2.3 E-UTRAN FDD - UE Timing Advance Adjustment Accuracy Test for 5MHz Bandwidth |  |  | Same as 7.2.1 |  |  | Same as 7.2.1 |  |  |  |  |  |  | Same as 7.2.1 |  |
| 7.2.4 E-UTRAN FDD - UE Timing Advance Adjustment Accuracy Test for SCell in sTAG |  |  | Same as 7.2.1 |  |  | Same as 7.2.1 |  |  |  |  |  |  | Same as 7.2.1 |  |
| 7.2.5 E-UTRAN TDD - UE Timing Advance Adjustment Accuracy Test for SCell in sTAG |  |  | Same as 7.2.1 |  |  | Same as 7.2.1 |  |  |  |  |  |  | Same as 7.2.1 |  |
| 7.2.5A E-UTRAN TDD - UE Timing Advance Adjustment Accuracy Test for SCell in sTAG for 20MHz +20MHz bandwidth |  |  | Same as 7.2.1 |  |  | Same as 7.2.1 |  |  |  |  |  |  | Same as 7.2.1 |  |
| 7.2.5B E-UTRAN TDD - UE Timing Advance Adjustment Accuracy Test for SCell in sTAG for 20MHz +10MHz bandwidth |  |  | Same as 7.2.1 |  |  | Same as 7.2.1 |  |  |  |  |  |  | Same as 7.2.1 |  |
| 7.2.6 E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA |  |  | Same as 7.2.1 |  |  | Same as 7.2.1 |  |  |  |  |  |  | Same as 7.2.1 |  |
| 7.2.7 E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA |  |  | Same as 7.2.1 |  |  | Same as 7.2.1 |  |  |  |  |  |  | Same as 7.2.1 |  |
| 7.2.8 E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA |  |  | Same as 7.2.1 |  |  | Same as 7.2.1 |  |  |  |  |  |  | Same as 7.2.1 |  |
| 7.2.9 HD-FDD UE Timing Advance Adjustment Accuracy Test for Category NB1 UE in Standalone Mode under Enhance Coverage |  |  | Timing Advance Adjustment: ±13.33Ts |  |  | 0.5 TS |  |  |  |  |  |  | Timing Advance Adjustment: ±13.83Ts |  |
| 7.2.10 E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB |  |  | Same as 7.2.1 |  |  | Same as 7.2.1 |  |  |  |  |  |  | Same as 7.2.1 |  |
| 7.2.11 E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB |  |  | Same as 7.2.1 |  |  | Same as 7.2.1 |  |  |  |  |  |  | Same as 7.2.1 |  |
| 7.2.12 E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB |  |  | Same as 7.2.1 |  |  | Same as 7.2.1 |  |  |  |  |  |  | Same as 7.2.1 |  |
| 7.2.13 E-UTRAN FDD - UE Timing Advance Adjustment Accuracy Test for sTTI and Short Processing Time |  |  | Same as 7.2.1 |  |  | Same as 7.2.1 |  |  |  |  |  |  | Same as 7.2.1 |  |
| 7.2.14 E-UTRAN TDD - UE Timing Advance Adjustment Accuracy Test for sTTI and Short Processing Time |  |  | Same as 7.2.1 |  |  | Same as 7.2.1 |  |  |  |  |  |  | Same as 7.2.1 |  |
| 7.2.15 E-UTRAN TDD - TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage |  |  | Same as 7.2.9 |  |  | Same as 7.2.9 |  |  |  |  |  |  | Same as 7.2.9 |  |
| 7.3.1 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync |  |  | SNRs as specified |  |  | 0.6dB (Subtests 1&2) 0.8dB (Subtest 3) 0.9dB (Subtest 4) |  |  |  |  |  |  | During T1: Formula: SNR + TT During T2: Formula: SNR + TT During T3: Formula: SNR - TT |  |
| 7.3.1_1 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync with 4 Rx antenna ports |  |  | SNRs as specified |  |  | Same as 7.3.1 |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.2 E-UTRAN FDD Radio Link Monitoring Test for In-sync |  |  | SNRs as specified |  |  | 0.8dB (Subtest 1) 0.9dB (Subtest 2) |  |  |  |  |  |  | During T1: Formula: SNR + TT During T2: Formula: SNR + TT During T3: Formula: SNR - TT During T4: Formula: SNR - TT S During T5: Formula: SNR + TT |  |
| 7.3.2_1 E-UTRAN FDD Radio Link Monitoring Test for In-sync with 4 Rx antenna ports |  |  | SNRs as specified |  |  | Same as 7.3.2 |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.3 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync |  |  | SNRs as specified |  |  | Same as 7.3.1 |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.3_1 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync with 4 Rx antenna ports |  |  | SNRs as specified |  |  | Same as 7.3.1 |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.4 E-UTRAN TDD Radio Link Monitoring Test for In-sync |  |  | SNRs as specified |  |  | Same as 7.3.2 |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.5 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX |  |  | SNRs as specified |  |  | 0.9dB (Subtest 1) 0.6dB (Subtest 2) |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.5_1 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX with 4 Rx antenna ports |  |  | SNRs as specified |  |  | Same as 7.3.5 |  |  |  |  |  |  | Same as 7.3.5 |  |
| 7.3.6 E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.6_1 E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX with 4 Rx antenna ports |  |  | SNRs as specified |  |  | Same as 7.3.6 |  |  |  |  |  |  | Same as 7.3.6 |  |
| 7.3.7 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX |  |  | SNRs as specified |  |  | 0.9dB (Subtest 1) 0.6dB (Subtest 2) |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.7_1 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX with 4 Rx antenna ports |  |  | SNRs as specified |  |  | Same as 7.3.7 |  |  |  |  |  |  | Same as 7.3.7 |  |
| 7.3.8 E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.8_! E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX with 4 Rx antenna ports |  |  | SNRs as specified |  |  | Same as 7.3.8 |  |  |  |  |  |  | Same as 7.3.8 |  |
| 7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non MBSFN ABS (eICIC) |  |  | SNRs as specified |  |  | 0.9dB for cell 1 (TT1) 0.2dB for cell 2 (TT2) |  |  |  |  |  |  | During T1: Cell 1 Formula: SNR + TT1 Cell 2 Formula: SNR - TT2 During T2: Cell 1 Formula: SNR + TT1 Cell 2 Formula: SNR - TT2 During T3: Cell 1 Formula: SNR - TT1 Cell 2 Formula: SNR + TT2 Formuale for Test Tolerance values: TT1: Ês1/Noc uncertainty TT2: (Ês2/Ês1 uncertainty - Ês1/Noc uncertainty) |  |
| 7.3.10 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non MBSFN ABS (eICIC) |  |  | Same as 7.3.9 |  |  | Same as 7.3.9 |  |  |  |  |  |  | Same as 7.3.9 |  |
| 7.3.11 E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with Non MBSFN ABS (eICIC) |  |  | SNRs as specified |  |  | 0.9dB for cell 1 (TT1) 0.2dB for cell 2(TT2) |  |  |  |  |  |  | During T1: Cell 1 Formula: SNR + TT1 Cell 2 Formula: SNR - TT2 During T2: Cell 1 Formula: SNR + TT1 Cell 2 Formula: SNR - TT2 During T3: Cell 1 Formula: SNR - TT1 Cell 2 Formula: SNR + TT2 During T4: Cell 1 Formula: SNR - TT1 Cell 2 Formula: SNR + TT2 During T5: Cell 1 Formula: SNR + TT1 Cell 2 Formula: SNR - TT2 Formuale for Test Tolerance values: TT1: Ês1/Noc uncertainty TT2: (Ês2/Ês1 uncertainty - Ês1/Noc uncertainty) |  |
| 7.3.12 E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with Non MBSFN ABS (eICIC) |  |  | Same as 7.3.11 |  |  | Same as 7.3.11 |  |  |  |  |  |  | Same as 7.3.11 |  |
| 7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) |  |  | Same as 7.3.9 |  |  | Same as 7.3.9 |  |  |  |  |  |  | Same as 7.3.9 |  |
| 7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) |  |  | Same as 7.3.9 |  |  | Same as 7.3.9 |  |  |  |  |  |  | Same as 7.3.9 |  |
| 7.3.15 E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) |  |  | Same as 7.3.11 |  |  | Same as 7.3.11 |  |  |  |  |  |  | Same as 7.3.11 |  |
| 7.3.16 E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) |  |  | Same as 7.3.11 |  |  | Same as 7.3.11 |  |  |  |  |  |  | Same as 7.3.11 |  |
| 7.3.17 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS assistance information and Non MBSFN ABS (feICIC) |  |  | SNRs as specified |  |  |  |  | 0.9dB for cell 1 (TT1) 0.2dB for cell 2 (TT2) 0.2dB for cell 3 (TT3) |  |  |  |  | During T1: Cell 1 Formula: SNR + TT1 Cell 2 Formula: SNR - TT2 Cell 3 Formula: SNR - TT3 During T2: Cell 1 Formula: SNR + TT1 Cell 2 Formula: SNR - TT2 Cell 3 Formula: SNR - TT3 During T3: Cell 1 Formula: SNR - TT1 Cell 2 Formula: SNR + TT2 Cell 3 Formula: SNR + TT3 Formuale for Test Tolerance values: TT1: Ês1/Noc uncertainty TT2: (Ês2/Ês1 uncertainty - Ês1/Noc uncertainty) TT3: (Ês3/Ês1 uncertainty - Ês1/Noc uncertainty) |  |
| 7.3.18 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS assistance information and Non MBSFN ABS (feICIC) |  |  | Same as 7.3.17 |  |  |  |  | Same as 7.3.17 |  |  |  |  | Same as 7.3.17 |  |
| 7.3.19 E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and Non MBSFN ABS (feICIC) |  |  | SNRs as specified |  |  |  |  | 0.9dB for cell 1 (TT1) 0.2dB for cell 2(TT2) 0.2dB for cell 3 (TT3) |  |  |  |  | During T1: Cell 1 Formula: SNR + TT1 Cell 2 Formula: SNR - TT2 Cell 3 Formula: SNR - TT3 During T2: Cell 1 Formula: SNR + TT1 Cell 2 Formula: SNR - TT2 Cell 3 Formula: SNR - TT3 During T3: Cell 1 Formula: SNR - TT1 Cell 2 Formula: SNR + TT2 Cell 3 Formula: SNR + TT3 During T4: Cell 1 Formula: SNR - TT1 Cell 2 Formula: SNR + TT2 Cell 3 Formula: SNR + TT3 During T5: Cell 1 Formula: SNR + TT1 Cell 2 Formula: SNR - TT2 Cell 3 Formula: SNR - TT3 Formuale for Test Tolerance values: TT1: Ês1/Noc uncertainty TT2: (Ês2/Ês1 uncertainty - Ês1/Noc uncertainty) TT3: (Ês3/Ês1 uncertainty - Ês1/Noc uncertainty) |  |
| 7.3.20 E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and Non MBSFN ABS (feICIC) |  |  | Same as 7.3.19 |  |  |  |  | Same as 7.3.19 |  |  |  |  | Same as 7.3.19 |  |
| 7.3.21 E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and MBSFN ABS (feICIC) |  |  | Same as 7.3.19 |  |  |  |  | Same as 7.3.19 |  |  |  |  | Same as 7.3.19 |  |
| 7.3.22 E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and MBSFN ABS (feICIC) |  |  | Same as 7.3.19 |  |  |  |  | Same as 7.3.19 |  |  |  |  | Same as 7.3.19 |  |
| 7.3.23 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth |  |  | SNRs as specified |  |  | Same as 7.3.1 |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.23_1 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth with 4 Rx antenna ports |  |  | SNRs as specified |  |  | Same as 7.3.23 |  |  |  |  |  |  | Same as 7.3.23 |  |
| 7.3.24 E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth |  |  | SNRs as specified |  |  | 0.9dB |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.24_1 E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth with 4 Rx antenna ports |  |  | SNRs as specified |  |  | Same as 7.3.24 |  |  |  |  |  |  | Same as 7.3.24 |  |
| 7.3.25 E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.25_1 E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth with 4 Rx antenna ports |  |  | SNRs as specified |  |  | Same as 7.3.25 |  |  |  |  |  |  | Same as 7.3.25 |  |
| 7.3.26 E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE category 0 |  |  | SNRs as specified |  |  | 0.9dB |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.26_2 E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 1bis |  |  | SNRs as specified |  |  | Same as 7.3.26 |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.27 E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE category 0 |  |  | SNRs as specified |  |  | Same as 7.3.26 |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.27_2 E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 1bis |  |  | SNRs as specified |  |  | Same as 7.3.26 |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.28 E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0 |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.28_2 E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE Category 1bis |  |  | SNRs as specified |  |  | Same as 7.3.28 |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.29 E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE category 0 |  |  | SNRs as specified |  |  | Same as 7.3.28 |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.29_2 E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 1bis |  |  | SNRs as specified |  |  | Same as 7.3.28 |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.30 E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE category 0 |  |  | SNRs as specified |  |  | Same as 7.3.26 |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.31 E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE category 0 |  |  | SNRs as specified |  |  | Same as 7.3.26 |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.32 E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0 |  |  | SNRs as specified |  |  | Same as 7.3.28 |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.33 E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE category 0 |  |  | SNRs as specified |  |  | Same as 7.3.28 |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.34 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE category 0 |  |  | SNRs as specified |  |  | Same as 7.3.26 |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.34_2 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 1bis |  |  | SNRs as specified |  |  | Same as 7.3.26 |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.35 E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0 |  |  | SNRs as specified |  |  | Same as 7.3.26 |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.35_2 E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE Category 1bis |  |  | SNRs as specified |  |  | Same as 7.3.26 |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.36 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0 |  |  | SNRs as specified |  |  | Same as 7.3.28 |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.36_2 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE Category 1bis |  |  | SNRs as specified |  |  | Same as 7.3.28 |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.37 E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0 |  |  | SNRs as specified |  |  | Same as 7.3.28 |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.37_2 E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category 1bis |  |  | SNRs as specified |  |  | Same as 7.3.28 |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.38 E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC |  |  | SNRs as specified |  |  | 0.9dB |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.38_1 E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with 4 Rx antenna ports |  |  | SNRs as specified |  |  | Same as 7.3.38 |  |  |  |  |  |  | Same as 7.3.38 |  |
| 7.3.39 E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC |  |  | SNRs as specified |  |  | 0.9dB |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.39 E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC with 4 Rx antenna ports |  |  | SNRs as specified |  |  | Same as 7.3.39 |  |  |  |  |  |  | Same as 7.3.39 |  |
| 7.3.40 E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC |  |  | SNRs as specified |  |  | 0.9dB |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.41 E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.42 E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.43 E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.44 E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD |  |  | SNRs as specified |  |  | 0.9dB |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.45 E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD |  |  | SNRs as specified |  |  | 0.9dB |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.46 E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.47 E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.48 E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A |  |  | SNRs as specified |  |  | 0.9dB |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.49 E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A |  |  | SNRs as specified |  |  | 0.9dB |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.50: E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-Sync in DRX for UE Category M1 Configured in CEMode A |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.51 E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE Category M1 Configured in CEMode A |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.52 E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A |  |  | SNRs as specified |  |  | 0.9dB |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.53 E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A |  |  | SNRs as specified |  |  | 0.9dB |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.54: E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE Category M1 Configured in CEMode A |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.55: E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.56:E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A |  |  | SNRs as specified |  |  | 0.9dB |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.57:E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A |  |  | SNRs as specified |  |  | 0.9dB |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.58:E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.59:E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.60 HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | During T1: Formula: SNR + TT During T2: Formula: SNR + TT During T3: Formula: SNR - TT During T4: Formula: SNR + TT |  |
| 7.3.61 HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.60 |  |
| 7.3.62 HD-FDD Radio Link Monitoring Test for In-sync with DRX for Category NB1 In-Band mode in Enhanced Coverage |  |  | SNRs as specified Time durations as specified |  |  | 0.6dB 0.28s, T2 only |  |  |  |  |  |  | During T1: Formula: SNR + TT T1 duration unchanged During T2: Formula: SNR - TT Formula: T2 duration -TT During T3: Formula: SNR + TT T3 duration unchanged |  |
| 7.3.63 HD-FDD Radio Link Monitoring Test for In-sync with DRX for Category NB1 In-Band mode in Normal Coverage |  |  | SNRs as specified Time durations as specified |  |  | 0.6dB 0.28s, T2 only |  |  |  |  |  |  | Same as 7.3.62 |  |
| 7.3.64 HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage |  |  | SNRs as specified Time durations as specified |  |  | 0.6dB 0.28s, T2 only |  |  |  |  |  |  | Same as 7.3.62 |  |
| 7.3.65 HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage |  |  | SNRs as specified Time durations as specified |  |  | 0.6dB 0.28s, T2 only |  |  |  |  |  |  | Same as 7.3.62 |  |
| 7.3.66 HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.60 |  |
| 7.3.67 HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.60 |  |
| 7.3.69 E-UTRAN HD-FDD Early Out-of-sync reporting Test for CE UE in CEMode A |  |  | SNRs as specified |  |  | 0.9dB |  |  |  |  |  |  | Same as 7.3.52 The Test Tolerance values are taken from the analysis for similar HD-FDD Test case 7.3.52 and no separate analysis is needed. |  |
| 7.3.88 TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage |  |  | Same as 7.3.60 |  |  | Same as 7.3.60 |  |  |  |  |  |  | Same as 7.3.60 |  |
| 7.3.89 TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage |  |  | Same as 7.3.61 |  |  | Same as 7.3.61 |  |  |  |  |  |  | Same as 7.3.61 |  |
| 7.3.90 TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage |  |  | Same as 7.3.28 |  |  | Same as 7.3.28 |  |  |  |  |  |  | Same as 7.3.28 |  |
| 7.3.91 TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage |  |  | Same as 7.3.28 |  |  | Same as 7.3.28 |  |  |  |  |  |  | Same as 7.3.28 |  |
| 7.3.92 TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage |  |  | Same as 7.3.64 |  |  | Same as 7.3.64 |  |  |  |  |  |  | Same as 7.3.64 |  |
| 7.3.93 TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage |  |  | Same as 7.3.65 |  |  | Same as 7.3.65 |  |  |  |  |  |  | Same as 7.3.65 |  |
| 7.3.94 TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage |  |  | Same as 7.3.64 |  |  | Same as 7.3.64 |  |  |  |  |  |  | Same as 7.3.64 |  |
| 7.3.95 TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage |  |  | Same as 7.3.64 |  |  | Same as 7.3.64 |  |  |  |  |  |  | Same as 7.3.64 |  |
| 7.4.1 E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Formula: SNR + TT |  |
| 7.4.2 E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.4.1 |  |
| 7.4.3 E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.4.1 |  |
| 7.5.1 E-UTRAN FDD - UE ProSe Direct Discovery Transmission Timing Accuracy Test |  |  | Uplink timing: ±12Ts Ês / Noc: +3.00dB |  |  | ±3Ts +0.3dB |  |  |  |  |  |  | Uplink timing: ±15Ts Ês / Noc: +3.30dB |  |
| 7.5.4 E-UTRAN FDD - UE ProSe Direct Communication Transmission Timing Accuracy Test |  |  | Uplink timing: ±12Ts Ês / Noc: +3.00dB |  |  | ±3Ts +0.3dB |  |  |  |  |  |  | Uplink timing: ±15Ts Ês / Noc: +3.30dB |  |
| 7.6.1 E-UTRAN FDD-TDD CA interruption at SRS carrier based switching |  |  | SNRs as specified |  |  | 0dB |  |  |  |  |  |  | Formula: SNR + TT |  |
| 7.6.2 E-UTRAN TDD-TDD CA interruption at SRS carrier based switching |  |  | SNRs as specified |  |  | 0dB |  |  |  |  |  |  | Same as 7.6.1 |  |
| 8.1.1 E-UTRAN FDD-FDD intra frequency event triggered reporting under fading propagation conditions in asynchronous cells |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |  |  | During T1: 0dB 2.10dB 0dB During T2: 0dB 2.10dB 2.10dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +6.10dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +6.10dB Ês2 / Noc: +6.10dB |  |
| 8.1.2 E-UTRAN FDD-FDD intra frequency event triggered reporting under fading propagation conditions in synchronous cells |  |  | Same as 8.1.1 |  |  | Same as 8.1.1 |  |  |  |  |  |  | Same as 8.1.1 |  |
| 8.1.3 E-UTRAN FDD-FDD intra frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX |  |  | Same as 8.1.1 |  |  | Same as 8.1.1 |  |  |  |  |  |  | Same as 8.1.1 |  |
| 8.1.5 E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB |  |
| 8.1.6 E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX |  |  | Same as 8.1.5 |  |  | Same as 8.1.5 |  |  |  |  |  |  | Same as 8.1.5 |  |
| 8.1.7 UE Measurement Procedures / E-UTRAN FDD-FDD Intra-frequency event-triggered reporting under time domain measurement resource restriction with non-MBSFN ABS (eICIC) |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +1.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +1.00dB Ês2 / Noc: -4.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.8dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +1.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +1.00dB Ês2 / Noc: -3.20dB |  |
| 8.1.8 E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +2.00dB Ês3 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +2.00dB Ês3 / Noc: -4.00dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0.8dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +2.00dB Ês3 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês1 / Noc: +2.00dB Ês2 / Noc: -3.20dB |  |
| 8.1.9 E-UTRAN FDD-FDD intra frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz Bandwidth |  |  | Same as 8.1.1 |  |  | Same as 8.1.1 |  |  |  |  |  |  | Same as 8.1.1 |  |
| 8.1.10 E-UTRAN FDD-FDD intra frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for 5MHz Bandwidth |  |  | Same as 8.1.3 |  |  | Same as 8.1.3 |  |  |  |  |  |  | Same as 8.1.3 |  |
| 8.1.11 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0 |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |  |  | During T1: 0dB 2.10dB 0dB During T2: 0dB 2.10dB 2.10dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +6.10dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +6.10dB Ês2 / Noc: +6.10dB |  |
| 8.1.11_2 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 1bis |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +7.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +7.00dB Ês2 / Noc: +4.00dB |  |  | During T1: 0dB 0.9dB 0dB During T2: 0dB 0.9dB 2.40dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +7.90dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +7.90dB Ês2 / Noc: +6.40dB |  |
| 8.1.12 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0 |  |  | Same as 8.1.11 |  |  | Same as 8.1.11 |  |  |  |  |  |  | Same as 8.1.11 |  |
| 8.1.12_2 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 1bis |  |  | Same as 8.1.11_2 |  |  | Same as 8.1.11_2 |  |  |  |  |  |  | Same as 8.1.11_2 |  |
| 8.1.13 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0 |  |  | Same as 8.1.11 |  |  | Same as 8.1.11 |  |  |  |  |  |  | Same as 8.1.11 |  |
| 8.1.13_2 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 1bis |  |  | Same as 8.1.11_2 |  |  | Same as 8.1.11_2 |  |  |  |  |  |  | Same as 8.1.11_2 |  |
| 8.1.17 E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0 |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |  |  | During T1: 0dB 2.60dB 0dB During T2: 0dB 2.60dB 2.60dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +6.60dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +6.60dB Ês2 / Noc: +6.60dB |  |
| 8.1.17_2 E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 1bis |  |  | Same as 8.1.11_2 |  |  | Same as 8.1.11_2 |  |  |  |  |  |  | Same as 8.1.11_2 |  |
| 8.1.18 E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0 |  |  | Same as 8.1.17 |  |  | Same as 8.1.17 |  |  |  |  |  |  | Same as 8.1.17 |  |
| 8.1.18_2 E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 1bis |  |  | Same as 8.1.11_2 |  |  | Same as 8.1.11_2 |  |  |  |  |  |  | Same as 8.1.11_2 |  |
| 8.1.19 E-UTRAN FD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0 |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB |  |
| 8.1.19_2 E-UTRAN FD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 1bis |  |  | Same as 8.1.19 |  |  | Same as 8.1.19 |  |  |  |  |  |  | Same as 8.1.19 |  |
| 8.1.20 E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0 |  |  | Same as 8.1.19 |  |  | Same as 8.1.19 |  |  |  |  |  |  | Same as 8.1.19 |  |
| 8.1.20_2 E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 1bis |  |  | Same as 8.1.20 |  |  | Same as 8.1.20 |  |  |  |  |  |  | Same as 8.1.20 |  |
| 8.1.21 E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0 |  |  | Same as 8.1.19 |  |  | Same as 8.1.19 |  |  |  |  |  |  | Same as 8.1.19 |  |
| 8.1.22 E-UTRAN HD- FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0 |  |  | Same as 8.1.19 |  |  | Same as 8.1.19 |  |  |  |  |  |  | Same as 8.1.19 |  |
| 8.1.23 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.2dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.20dB |  |
| 8.1.24 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA |  |  | Same as 8.1.23 |  |  | Same as 8.1.23 |  |  |  |  |  |  | Same as 8.1.23 |  |
| 8.1.25 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX |  |  | Same as 8.1.23 |  |  | Same as 8.1.23 |  |  |  |  |  |  | Same as 8.1.23 |  |
| 8.1.26 E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA |  |  | Same as 8.1.23 |  |  | Same as 8.1.23 |  |  |  |  |  |  | Same as 8.1.23 |  |
| 8.1.27 E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA |  |  | Same as 8.1.23 |  |  | Same as 8.1.23 |  |  |  |  |  |  | Same as 8.1.23 |  |
| 8.1.28 E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX |  |  | Same as 8.1.23 |  |  | Same as 8.1.23 |  |  |  |  |  |  | Same as 8.1.23 |  |
| 8.1.29 E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA |  |  | Same as 8.1.23 |  |  | Same as 8.1.23 |  |  |  |  |  |  | Same as 8.1.23 |  |
| 8.1.30 E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX |  |  | Same as 8.1.23 |  |  | Same as 8.1.23 |  |  |  |  |  |  | Same as 8.1.23 |  |
| 8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB |  |  | Same as 8.1.33 |  |  | Same as 8.1.33 |  |  |  |  |  |  | Same as 8.1.33 |  |
| 8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB |  |  | Same as 8.1.33 |  |  | Same as 8.1.33 |  |  |  |  |  |  | Same as 8.1.33 |  |
| 8.1.33 E-UTRAN HD-FDD Intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: -12.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -12.00dB Ês2 / Noc: -12.00dB |  |  | During T1: 0dB 0.8dB 0dB During T2: 0dB 1.2dB 1.2dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: -11.20dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -10.80dB Ês2 / Noc: -10.80dB |  |
| 8.1.34 E-UTRAN HD-FDD Intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB |  |  | Same as 8.1.33 |  |  | Same as 8.1.33 |  |  |  |  |  |  | Same as 8.1.33 |  |
| 8.1.35 E-UTRAN TDD Intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB |  |  | Same as 8.1.33 |  |  | Same as 8.1.33 |  |  |  |  |  |  | Same as 8.1.33 |  |
| 8.1.40 E-UTRAN FDD-FDD intra-frequency event triggered reporting for UE configured with highSpeedEnhancedMeasFlag in synchronous cells |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB |  |  | During T1: 0dB 0.7dB 0dB During T2: 0dB 0.7dB 0.7dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.7dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.7dB Ês2 / Noc: +4.7dB |  |
| 8.1.41 E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for CE UE in CEModeA without gap |  |  | During T1: Noc: -104dBm/15kHz Ês1 / Noc: +19dB During T2: Noc: -104dBm/15kHz Ês / Noc: -3dB |  |  | During T1: 0dB 0.6dB During T2: 0dB 0dB |  |  |  |  |  |  | During T1: Noc: -104dBm/15kHz Ês1 / Noc: +19.6dB During T2: Noc: -104dBm/15kHz Ês / Noc: -3dB |  |
| 8.1.42 E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for CE UE in CEModeA without gap |  |  | Same as 8.1.41 |  |  | Same as 8.1.41 |  |  |  |  |  |  | Same as 8.1.41 |  |
| 8.2.1 E-UTRAN TDD-TDD intra frequency event triggered reporting under fading propagation conditions in synchronous cells |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |  |  | During T1: 0dB 2.10dB 0dB During T2: 0dB 2.60dB 2.60dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +6.10dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +6.60dB Ês2 / Noc: +6.60dB |  |
| 8.2.2 E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX |  |  | Same as 8.2.1 |  |  | Same as 8.2.1 |  |  |  |  |  |  | Same as 8.2.1 |  |
| 8.2.3 E-UTRAN TDD-TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB |  |
| 8.2.4 E-UTRAN TDD-TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX |  |  | Same as 8.2.3 |  |  | Same as 8.2.3 |  |  |  |  |  |  | Same as 8.2.3 |  |
| 8.2.5 UE Measurement Procedures / E-UTRAN TDD-TDD Intra-frequency event-triggered reporting under time domain measurement resource restriction with non-MBSFN ABS (eICIC) |  |  | Same as 8.1.7 |  |  | Same as 8.1.7 |  |  |  |  |  |  | Same as 8.1.7 |  |
| 8.2.6 E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) |  |  | Same as 8.1.8 |  |  | Same as 8.1.8 |  |  |  |  |  |  | Same as 8.1.8 |  |
| 8.2.7 E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0 |  |  | Same as 8.1.19 |  |  | Same as 8.1.19 |  |  |  |  |  |  | Same as 8.1.19 |  |
| 8.2.7_2 E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 1bis |  |  | Same as 8.2.7 |  |  | Same as 8.2.7 |  |  |  |  |  |  | Same as 8.2.7 |  |
| 8.2.8 E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0 |  |  | Same as 8.1.19 |  |  | Same as 8.1.19 |  |  |  |  |  |  | Same as 8.1.19 |  |
| 8.2.8_2 E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 1bis |  |  | Same as 8.2.8 |  |  | Same as 8.2.8 |  |  |  |  |  |  | Same as 8.2.8 |  |
| 8.2.11 E-UTRAN TDD-TDD intra-frequency event triggered reporting for UE configured with highSpeedEnhancedMeasFlag in synchronous cells |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |  |  | During T1: 0dB 0.9dB 0dB During T2: 0dB 0.9dB 0.9dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.90dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.90dB Ês2 / Noc: +4.90dB |  |
| 8.2.14 E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for CE in CEModeA without gap |  |  | Same as 8.1.41 |  |  | Same as 8.1.41 |  |  |  |  |  |  | Same as 8.1.41 |  |
| 8.3.1 E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0.7dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.70dB |  |
| 8.3.1_2 E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 1bis |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 1.7dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +8.70dB |  |
| 8.3.2 E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells |  |  | Same as 8.3.1 |  |  | Same as 8.3.1 |  |  |  |  |  |  | Same as 8.3.1 |  |
| 8.3.2_2 E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells for UE category 1bis |  |  | Same as 8.3.1_2 |  |  | Same as 8.3.1_2 |  |  |  |  |  |  | Same as 8.3.1_2 |  |
| 8.3.3 E-UTRAN FDD-FDD Inter frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +4.00dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +24.00dB |  |  | During T1: +2.5dB 0dB 0dB 0dB During T2: +2.5dB -4.0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc1: -95.50dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +4.00dB During T2: Noc1: -95.50dBm/15kHz Ês1 / Noc1: +0.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +24.00dB |  |
| 8.3.3_2 E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used for UE category 1bis |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +4.00dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +24.00dB |  |  | During T1: +2.5dB 0dB 0dB 0dB During T2: +2.5dB -4.0dB 0dB +1dB |  |  |  |  |  |  | During T1: Noc1: -95.50dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +4.00dB During T2: Noc1: -95.50dBm/15kHz Ês1 / Noc1: +0.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +25.00dB |  |
| 8.3.4 E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB |  |
| 8.3.5 E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX |  |  | Same as 8.3.4 |  |  | Same as 8.3.4 |  |  |  |  |  |  | Same as 8.3.4 |  |
| 8.3.6 E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB |  |
| 8.3.7 E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity Noc3: -98dBm/15kHz Ês3 / Noc3: -infinity Noc4: -98dBm/15kHz Ês4 / Noc4: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB Noc3: -98dBm/15kHz Ês3 / Noc3: +7.00dB Noc4: -98dBm/15kHz Ês4 / Noc4: +7.00dB |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity Noc3: -98dBm/15kHz Ês3 / Noc3: -infinity Noc4: -98dBm/15kHz Ês4 / Noc4: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB Noc3: -98dBm/15kHz Ês3 / Noc3: +7.00dB Noc4: -98dBm/15kHz Ês4 / Noc4: +7.00dB |  |
| 8.3.8 FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, non DRX |  |  | Same as 8.3.7 |  |  | Same as 8.3.7 |  |  |  |  |  |  | Same as 8.3.7 |  |
| 8.3.9 FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity Noc3: -98dBm/15kHz Ês3 / Noc3: -infinity Noc4: -98dBm/15kHz Ês4 / Noc4: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +4.00dB Noc3: -98dBm/15kHz Ês3 / Noc3: +4.00dB Noc4: -98dBm/15kHz Ês4 / Noc4: +4.00dB |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 1dB 0dB 1dB 0dB 1dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity Noc3: -98dBm/15kHz Ês3 / Noc3: -infinity Noc4: -98dBm/15kHz Ês4 / Noc4: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +5.00dB Noc3: -98dBm/15kHz Ês3 / Noc3: +5.00dB Noc4: -98dBm/15kHz Ês4 / Noc4: +5.00dB |  |
| 8.3.12 E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeA |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +4.00dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 4.65dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +8.65dB |  |
| 8.3.13 E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeA |  |  | Same as 8.3.12 |  |  | Same as 8.3.12 |  |  |  |  |  |  | Same as 8.3.12 |  |
| 8.3.14 E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeB |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -12.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -12.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -12.00dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 4.65dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -12.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -12.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -8.35dB |  |
| 8.3.15 E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeB |  |  | Same as 8.3.14 |  |  | Same as 8.3.14 |  |  |  |  |  |  | Same as 8.3.14 |  |
| 8.3.16 E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeA when DRX is used |  |  | Same as 8.3.12 |  |  | Same as 8.3.12 |  |  |  |  |  |  | Same as 8.3.12 |  |
| 8.3.17 E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeA when DRX is used |  |  | Same as 8.3.12 |  |  | Same as 8.3.12 |  |  |  |  |  |  | Same as 8.3.12 |  |
| 8.3.18 E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeB when DRX is used |  |  | Same as 8.3.14 |  |  | Same as 8.3.14 |  |  |  |  |  |  | Same as 8.3.14 |  |
| 8.3.19 E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeB when DRX is used |  |  | Same as 8.3.14 |  |  | Same as 8.3.14 |  |  |  |  |  |  | Same as 8.3.14 |  |
| 8.4.1 E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0.7dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.70dB |  |
| 8.4.1_2 E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 1bis |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 1.7dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +8.70dB |  |
| 8.4.2 E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells |  |  | Same as 8.4.1 |  |  | Same as 8.4.1 |  |  |  |  |  |  | Same as 8.4.1 |  |
| 8.4.2_2 E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells for UE category 1bis |  |  | Same as 8.4.1_2 |  |  | Same as 8.4.1_2 |  |  |  |  |  |  | Same as 8.4.1_2 |  |
| 8.4.3 E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used |  |  | Same as 8.3.3 |  |  | Same as 8.3.3 |  |  |  |  |  |  | Same as 8.3.3 |  |
| 8.4.3_2 E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used for UE category 1bis |  |  | Same as 8.3.3_2 |  |  | Same as 8.3.3_2 |  |  |  |  |  |  | Same as 8.3.3_2 |  |
| 8.4.4 E-UTRAN TDD-TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB |  |
| 8.4.5 E-UTRAN TDD-TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX |  |  | Same as 8.4.4 |  |  | Same as 8.4.4 |  |  |  |  |  |  | Same as 8.4.4 |  |
| 8.4.6 E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0 |  |  | Same as 8.4.1 |  |  | Same as 8.4.1 |  |  |  |  |  |  | Same as 8.4.1 |  |
| 8.4.6_2 E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0 for UE category 1bis |  |  | Same as 8.4.1_2 |  |  | Same as 8.4.1_2 |  |  |  |  |  |  | Same as 8.4.1_2 |  |
| 8.4.7 E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group |  |  | Same as 8.3.7 |  |  | Same as 8.3.7 |  |  |  |  |  |  | Same as 8.3.7 |  |
| 8.4.8 TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX |  |  | Same as 8.3.8 The Test Tolerance values are taken from the analysis for similar FDD-FDD Test case 8.3.8 and no separate analysis is needed. |  |  | Same as 8.3.9 The Test Tolerance values are taken from the analysis for similar FDD-FDD Test case 8.3.8 and no separate analysis is needed. |  |  |  |  |  |  | Same as 8.3.8 The Test Tolerance values are taken from the analysis for similar FDD-FDD Test case 8.3.8 and no separate analysis is needed. |  |
| 8.4.9 TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX |  |  | Same as 8.3.9 The Test Tolerance values are taken from the analysis for similar FDD-FDD Test case 8.3.9 and no separate analysis is needed. |  |  | Same as 8.3.9 The Test Tolerance values are taken from the analysis for similar FDD-FDD Test case 8.3.9 and no separate analysis is needed. |  |  |  |  |  |  | Same as 8.3.9 The Test Tolerance values are taken from the analysis for similar FDD-FDD Test case 8.3.9 and no separate analysis is needed. |  |
| 8.4.12 E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeA |  |  | Same as 8.3.12 The Test Tolerance values are taken from the analysis for similar FDD Test case 8.3.12 and no separate analysis is needed. |  |  | Same as 8.3.12 The Test Tolerance values are taken from the analysis for similar FDD Test case 8.3.12 and no separate analysis is needed. |  |  |  |  |  |  | Same as 8.3.12 The Test Tolerance values are taken from the analysis for similar FDD Test case 8.3.12 and no separate analysis is needed. |  |
| 8.4.13 E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeB |  |  | Same as 8.3.13 The Test Tolerance values are taken from the analysis for similar FDD Test case 8.3.13 and no separate analysis is needed. |  |  | Same as 8.3.12 The Test Tolerance values are taken from the analysis for similar FDD Test case 8.3.13 and no separate analysis is needed. |  |  |  |  |  |  | Same as 8.3.12 The Test Tolerance values are taken from the analysis for similar FDD Test case 8.3.13 and no separate analysis is needed. |  |
| 8.4.14 TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeA when DRX is used |  |  | Same as 8.4.12 |  |  | Same as 8.4.12 |  |  |  |  |  |  | Same as 8.4.12 |  |
| 8.4.15 E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeB when DRX is used |  |  | Same as 8.4.13 |  |  | Same as 8.4.13 |  |  |  |  |  |  | Same as 8.4.13 |  |
| 8.5.1 E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -1.8 dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -1.8dB |  |
| 8.5.2 E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -3.35 dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0.4dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -2.95dB |  |
| 8.5.3 E-UTRAN FDD - UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions |  |  | Same as 8.5.1 |  |  |  |  | Same as 8.5.1 |  |  |  |  | Same as 8.5.1 |  |
| 8.5.4 E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: 0.02 dB |  |  |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: 0.02dB |  |
| 8.5.6 E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -1.8 dB |  |  |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -1.8dB |  |
| 8.5.7 E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions for 5MHz Bandwidth |  |  | Same as 8.5.1 |  |  |  |  | Same as 8.5.1 |  |  |  |  | Same as 8.5.1 |  |
| 8.5.8 E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cells 2, 3 and 4 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1: Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cells 2, 3 and 4 Ioc: -70dBm/3.84MHz Ior / Ioc: -1.8 dB |  |  |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cells 2, 3 and 4 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cells 2, 3 and 4 Ioc: -70dBm/3.84MHz Ior / Ioc: -1.8 dB |  |
| 8.6.1 E-UTRAN TDD -UTRAN FDD event triggered reporting under fading propagation conditions |  |  | Same as 8.5.1 |  |  | Same as 8.5.1 |  |  |  |  |  |  | Same as 8.5.1 |  |
| 8.6.3 E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX |  |  | Same as 8.5.8 |  |  | Same as 8.5.8 |  |  |  |  |  |  | Same as 8.5.8 |  |
| 8.7.1 E-UTRAN TDD - UTRAN TDD event triggered reporting under fading propagation conditions |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +9dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: -inf PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +9dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: +5dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +9dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: -inf PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +9dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: +5dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |
| 8.7.2 E-UTRAN TDD - UTRAN TDD cell search when DRX is used under fading propagation conditions |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: -inf PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: +9dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |  | During T1: 0dB 0dB -0.40dB 0dB 0dB 0dB During T2: 0dB 0dB -0.40dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cell 2 Ioc: -80.40dBm/1.28MHz Ior / Ioc: -inf PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cell 2 Ioc: -80.40dBm/1.28MHz Ior / Ioc: +9dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |
| 8.7.3 E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting under AWGN propagation conditions |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cell 2 Ioc: -75dBm/1.28MHz Ior / Ioc: -inf PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cell 2 Ioc: -75dBm/1.28MHz Ior / Ioc: +5dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cell 2 Ioc: -75dBm/1.28MHz Ior / Ioc: -inf PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cell 2 Ioc: -75dBm/1.28MHz Ior / Ioc: +5dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |
| 8.7.4 E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: - infinity PCCPCH_Ec/Ior: -4.77dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: +8dB PCCPCH_Ec/Ior: -4.77dB DwPCH_Ec/Ior: 0dB |  |  | During T1: 0dB 0dB +0.6dB 0dB 0dB 0dB During T2: 0dB 0dB +0.6dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cell 2 Ioc: -79.4dBm/1.28MHz Ior / Ioc: - infinity PCCPCH_Ec/Ior: -4.77dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cell 2 Ioc: -79.4dBm/1.28MHz Ior / Ioc: +8dB PCCPCH_Ec/Ior: -4.77dB DwPCH_Ec/Ior: 0dB |  |
| 8.7.5 E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cells 2, 3 and 4 Ioc: -70dBm/1.28MHz Ior / Ioc: -inf PCCPCH_Ec/Ior: -inf DwPCH_Ec/Ior: -inf During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cells 2,3 and 4 Ioc: -70dBm/1.28MHz Ior / Ioc: +9dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cells 2, 3 and 4 Ioc: -70dBm/1.28MHz Ior / Ioc: -inf PCCPCH_Ec/Ior: -inf DwPCH_Ec/Ior: -inf During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cells 2,3 and 4 Ioc: -70dBm/1.28MHz Ior / Ioc: +9dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |
| 8.7A.1 E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX |  |  | Same as 8.7.5 |  |  | Same as 8.7.5 |  |  |  |  |  |  | Same as 8.7.5 |  |
| 8.8.1 E-UTRAN FDD - GSM event triggered reporting in AWGN |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -75 dBm |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -75 dBm |  |
| 8.8.2 E-UTRAN FDD- GSM event triggered reporting when DRX is used in AWGN |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB GSM Cell 2 Signal level: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB GSM Cell 2 Signal level: -75dBm |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRAN Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB GSM Cell 2 Signal level: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB GSM Cell 2 Signal level: -75dBm |  |
| 8.9.1 E-UTRAN FDD - UTRAN TDD event triggered reporting under fading propagation conditions |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Iot: 4dB UTRA Cell 2 Ioc: -70dBm/1.28MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Iot: 4dB UTRA Cell 2 Ioc: -70dBm/1.28MHz Ior / Ioc: 9 dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0 dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Iot: 4dB UTRA Cell 2 Ioc: -70dBm/1.28MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Iot: 4dB UTRA Cell 2 Ioc: -70dBm/1.28MHz Ior / Ioc: 9 dB |  |
| 8.9.2 E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions |  |  | Same as 8.7.4 |  |  | Same as 8.7.4 |  |  |  |  |  |  | Same as 8.7.4 |  |
| 8.10.1 E-UTRAN TDD - GSM event triggered reporting in AWGN |  |  | Same as 8.8.1 |  |  | Same as 8.8.1 |  |  |  |  |  |  | Same as 8.8.1 |  |
| 8.10.2 E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN |  |  | Same as 8.8.2 |  |  | Same as 8.8.2 |  |  |  |  |  |  | Same as 8.8.2 |  |
| 8.11.1 Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 0dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: -infinity E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 0dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: 3dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: 3dB |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB +0.6dB 0dB +0.6dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 0dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: -infinity E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 0dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: 3.6dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: 3.6dB |  |
| 8.11.2 E-UTRAN TDD - E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions |  |  | Same as 8.11.1 |  |  | Same as 8.11.1 |  |  |  |  |  |  | Same as 8.11.1 |  |
| 8.11.3 E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: -infinity UTRA Cell 3 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: +7.00dB UTRA Cell 3 Ioc: -70dBm/3.84MHz Ior / Ioc: -1.8 dB |  |  | During T1: 0dB -1.05dB 0dB 0dB 0dB 0dB During T2: 0dB -1.05dB 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +2.95dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: -infinity UTRA Cell 3 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +2.95dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: +7.00dB UTRA Cell 3 Ioc: -70dBm/3.84MHz Ior / Ioc: -1.8 dB |  |
| 8.11.4 InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: -infinity UTRA Cell 3 Ioc: -80dBm/1.28MHz Ior / Ioc: -infinity PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: +7.00dB UTRA Cell 3 Ioc: -80dBm/1.28MHz Ior / Ioc: 9.00dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |  |  |  | During T1: 0dB -1.05dB 0dB 0dB -0.4dB 0dB 0dB 0dB During T2: 0dB -1.05dB 0dB 0dB -0.4dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +2.95dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: -infinity UTRA Cell 3 Ioc: -80.4dBm/1.28MHz Ior / Ioc: -infinity PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +2.95dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: +7.00dB UTRA Cell 3 Ioc: -80.4dBm/1.28MHz Ior / Ioc: 9.00dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |
| 8.11.5 Combined E-UTRAN FDD - E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: -infinity GSM Cell 3 Signal level: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: +7.00dB GSM Cell 3 Signal level: -75dBm |  |  |  |  | During T1: 0dB -0.2dB 0dB 0dB 0dB During T2: 0dB -0.2dB 0dB +0.5dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +3.80dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: -infinity GSM Cell 3 Signal level: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +3.80dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: +7.50dB GSM Cell 3 Signal level: -75dBm |  |
| 8.11.6 Combined E-UTRAN TDD - E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions |  |  | Same as 8.11.5 |  |  |  |  | Same as 8.11.5 |  |  |  |  | Same as 8.11.5 |  |
| 8.14.1 E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells |  |  | Same as 8.3.1 |  |  |  |  | Same as 8.3.1 |  |  |  |  | Same as 8.3.1 |  |
| 8.14.2 E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells |  |  | Same as 8.3.1 |  |  |  |  | Same as 8.3.1 |  |  |  |  | Same as 8.3.1 |  |
| 8.14.3 E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps |  |  | Same as 8.4.4 |  |  |  |  | Same as 8.4.4 |  |  |  |  | Same as 8.4.4 |  |
| 8.15.1 E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells |  |  | Same as 8.4.1 |  |  |  |  | Same as 8.4.1 |  |  |  |  | Same as 8.4.1 |  |
| 8.15.2 E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells |  |  | Same as 8.4.1 |  |  |  |  | Same as 8.4.1 |  |  |  |  | Same as 8.4.1 |  |
| 8.15.3 E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps |  |  | Same as 8.4.4 |  |  |  |  | Same as 8.4.4 |  |  |  |  | Same as 8.4.4 |  |
| 8.16.1 E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX |  |  | During T1: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 19dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 19dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 19dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 19dB E-UTRA Cell 3 Ês3 / Noc2: 19dB During T3: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: -3dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: -3dB E-UTRA Cell 3 Ês3 / Noc2: -3dB |  |  |  |  | During T1: 0dB +0.2dB 0dB +0.2dB 0dB During T2: 0dB +0.2dB 0dB +0.2dB 0dB During T3: 0dB 0dB 0dB -0.2dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 19.2dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 19.2dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 19.2dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 19.2dB E-UTRA Cell 3 Ês3 / Noc2: 19.0dB During T3: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: -3.0dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: -3.2dB E-UTRA Cell 3 Ês3 / Noc2: -3.0dB |  |
| 8.16.2 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX |  |  | Same as 8.16.1 |  |  |  |  | Same as 8.16.1 |  |  |  |  | Same as 8.16.1 |  |
| 8.16.3 E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX |  |  | During T1: E-UTRA Cell 1 Noc1: -98dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -98dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -98dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -98dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Ês3 / Noc2: 16dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -98dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -98dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -98dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -98dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Ês3 / Noc2: 16dB |  |
| 8.16.4 E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX |  |  | Same as 8.16.3 |  |  |  |  | Same as 8.16.3 |  |  |  |  | Same as 8.16.3 |  |
| 8.16.5 E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 19dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 19dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 19dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 19dB E-UTRA Cell 3 Ês3 / Noc2: 19dB During T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: -3dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -3dB E-UTRA Cell 3 Ês3 / Noc2: -3dB |  |  |  |  | During T1: 0dB +0.2dB -0.1dB +0.3dB 0dB During T2: 0dB +0.2dB -0.1dB +0.3dB 0dB During T3: 0dB 0dB -0.1dB -0.1dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 19.2dB E-UTRA Cell 2 Noc2: -104.1dBm/15kHz Ês2 / Noc2: 19.3dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 19.2dB E-UTRA Cell 2 Noc2: -104.1dBm/15kHz Ês2 / Noc2: 19.3dB E-UTRA Cell 3 Ês3 / Noc2: 19.0dB During T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: -3.0dB E-UTRA Cell 2 Noc2: -104.1dBm/15kHz Ês2 / Noc2: -3.1dB E-UTRA Cell 3 Ês3 / Noc2: -3.0dB |  |
| 8.16.6 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth |  |  | Same as 8.16.5 |  |  |  |  | Same as 8.16.5 |  |  |  |  | Same as 8.16.5 |  |
| 8.16.7 E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth |  |  | During T1: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Ês3 / Noc2: 16dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Ês3 / Noc2: 16dB |  |
| 8.16.8 E-UTRAN TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth |  |  | Same as 8.16.7 |  |  |  |  | Same as 8.16.7 |  |  |  |  | Same as 8.16.7 |  |
| 8.16.9 E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz |  |  | During T1: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 19dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 19dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 19dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 19dB E-UTRA Cell 3 Ês3 / Noc2: 19dB During T3: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: -3dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: -3dB E-UTRA Cell 3 Ês3 / Noc2: -3dB |  |  |  |  | During T1: 0dB +0.2dB 0dB +0.2dB 0dB During T2: 0dB +0.2dB 0dB +0.2dB 0dB During T3: 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 19.2dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 19.2dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 19.2dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 19.2dB E-UTRA Cell 3 Ês3 / Noc2: 19.0dB During T3: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: -3.0dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: -3.0dB E-UTRA Cell 3 Ês3 / Noc2: -3.0dB |  |
| 8.16.10 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz |  |  | Same as 8.16.9 |  |  |  |  | Same as 8.16.9 |  |  |  |  | Same as 8.16.9 |  |
| 8.16.11 E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz |  |  | Same as 8.16.3 |  |  |  |  | Same as 8.16.3 |  |  |  |  | Same as 8.16.3 |  |
| 8.16.12 E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz |  |  | Same as 8.16.3 |  |  |  |  | Same as 8.16.3 |  |  |  |  | Same as 8.16.3 |  |
| 8.16.13 E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz+5MHz |  |  | During T1: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 19dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 19dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 19dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 19dB E-UTRA Cell 3 Ês3 / Noc2: 19dB During T3: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: -3dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: -3dB E-UTRA Cell 3 Ês3 / Noc2: -3dB |  |  |  |  | During T1: 0dB +0.2dB 0dB +0.2dB 0dB During T2: 0dB +0.2dB 0dB +0.2dB 0dB During T3: 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 19.2dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 19.2dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 19.2dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 19.2dB E-UTRA Cell 3 Ês3 / Noc2: 19.0dB During T3: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: -3.0dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: -3.0dB E-UTRA Cell 3 Ês3 / Noc2: -3.0dB |  |
| 8.16.14 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5MHz+5MHz |  |  | Same as 8.16.13 |  |  |  |  | Same as 8.16.13 |  |  |  |  | Same as 8.16.13 |  |
| 8.16.15 E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 5MHz+5MHz |  |  | Same as 8.16.3 |  |  |  |  | Same as 8.16.3 |  |  |  |  | Same as 8.16.3 |  |
| 8.16.16 E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 5MHz+5MHz |  |  | Same as 8.16.3 |  |  |  |  | Same as 8.16.3 |  |  |  |  | Same as 8.16.3 |  |
| 8.16.17 E-UTRAN FDD activation and deactivation of known SCell in non-DRX |  |  | Same as 8.16.37 |  |  |  |  | Same as 8.16.37 |  |  |  |  | Same as 8.16.37 |  |
| 8.16.17A E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz |  |  | Same as 8.16.17 |  |  |  |  | Same as 8.16.17 |  |  |  |  | Same as 8.16.17 |  |
| 8.16.18 E-UTRAN TDD activation and deactivation of known SCell in non-DRX |  |  | Same as 8.16.17 |  |  |  |  | Same as 8.16.17 |  |  |  |  | Same as 8.16.17 |  |
| 8.16.18A E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz |  |  | Same as 8.16.17 |  |  |  |  | Same as 8.16.17 |  |  |  |  | Same as 8.16.17 |  |
| 8.16.21 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz bandwidth |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 19dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 19dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 19dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 19dB E-UTRA Cell 3 Ês3 / Noc2: 19dB During T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: -3dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -3dB E-UTRA Cell 3 Ês3 / Noc2: -3dB |  |  |  |  | During T1: 0dB +0.2dB 0dB +0.2dB 0dB During T2: 0dB +0.2dB 0dB +0.2dB 0dB During T3: 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 19.2dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 19.2dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 19.2dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 19.2dB E-UTRA Cell 3 Ês3 / Noc2: 19.0dB During T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: -3.0dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -3.0dB E-UTRA Cell 3 Ês3 / Noc2: -3.0dB |  |
| 8.16.22 E-UTRAN TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20MHz+10MHz bandwidth |  |  | Same as 8.16.3 |  |  |  |  | Same as 8.16.3 |  |  |  |  | Same as 8.16.3 |  |
| 8.16.23 E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Ês3 / Noc2: 17dB During T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: -3dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -3dB E-UTRA Cell 3 Ês3 / Noc2: -3dB |  |  |  |  | During T1: 0dB +0.2dB -0.5dB +0.7dB 0dB During T2: 0dB +0.2dB -0.5dB +0.7dB 0dB During T3: 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17.2dB E-UTRA Cell 2 Noc2: -104.5dBm/15kHz Ês2 / Noc2: 17.7dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17.2dB E-UTRA Cell 2 Noc2: -104.5dBm/15kHz Ês2 / Noc2: 17.7dB E-UTRA Cell 3 Ês3 / Noc2: 17.0dB During T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: -3.0dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -3.0dB E-UTRA Cell 3 Ês3 / Noc2: -3.0dB |  |
| 8.16.24 E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD |  |  | Same as 8.16.23 |  |  |  |  | Same as 8.16.23 |  |  |  |  | Same as 8.16.23 |  |
| 8.16.25 E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD |  |  | During T1: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Ês3 / Noc2: 16dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Ês3 / Noc2: 16dB |  |
| 8.16.26 E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD |  |  | Same as 8.16.25 |  |  |  |  | Same as 8.16.25 |  |  |  |  | Same as 8.16.25 |  |
| 8.16.27 E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with PCell in FDD |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -infinity E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Ês4 / Noc3: -infinity During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Ês4 / Noc3: 17dB During T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: -3dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -3dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: -3dB E-UTRA Cell 4 Ês4 / Noc3: -3dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB -0.5dB +0.7dB 0dB During T2: 0dB 0dB 0dB +0.2dB -0.5dB +0.7dB 0dB During T3: 0dB 0dB 0dB -0.2dB -0.5dB -0.7dB -0.7dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -infinity E-UTRA Cell 3 Noc3: -104.5dBm/15kHz Ês3 / Noc3: 17.7dB E-UTRA Cell 4 Ês4 / Noc3: -infinity During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17.2dB E-UTRA Cell 3 Noc3: -104.5dBm/15kHz Ês3 / Noc3: 17.7dB E-UTRA Cell 4 Ês4 / Noc3: 17dB During T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: -3dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -3.2dB E-UTRA Cell 3 Noc3: -104.5dBm/15kHz Ês3 / Noc3: -3.7dB E-UTRA Cell 4 Ês4 / Noc3: -3.7dB |  |
| 8.16.28 E-UTRAN TDD-FDD 3DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with PCell in TDD |  |  | Same as 8.16.27 |  |  |  |  | Same as 8.16.27 |  |  |  |  | Same as 8.16.27 |  |
| 8.16.29 3DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX |  |  | Same as 8.16.27 |  |  |  |  | Same as 8.16.27 |  |  |  |  | Same as 8.16.27 |  |
| 8.16.30 3DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX |  |  | Same as 8.16.27 |  |  |  |  | Same as 8.16.27 |  |  |  |  | Same as 8.16.27 |  |
| 8.16.31 E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD |  |  | During T1: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Ês4 / Noc3: -infinity During T2: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Ês4 / Noc3: 16dB During T3: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3:16dB E-UTRA Cell 4 Ês4 / Noc3: -infinity During T4: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Ês4 / Noc3: 16dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16 E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Ês4 / Noc3: -infinity During T2: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Ês4 / Noc3: 16dB During T3: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Ês4 / Noc3: -infinity E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Ês4 / Noc3: 16dB |  |
| 8.16.32 E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD |  |  | Same as 8.16.31 |  |  |  |  | Same as 8.16.31 |  |  |  |  | Same as 8.16.31 |  |
| 8.16.33 E-UTRAN FDD 3DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX |  |  | Same as 8.16.31 |  |  |  |  | Same as 8.16.31 |  |  |  |  | Same as 8.16.31 |  |
| 8.16.34 E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX |  |  | Same as 8.16.31 |  |  |  |  | Same as 8.16.31 |  |  |  |  | Same as 8.16.31 |  |
| 8.16.35 3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX |  |  | During T1, T2, T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB |  |  | During T1,T2,T3: 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1, T2, T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB |  |
| 8.16.36 3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX |  |  | Same as 8.16.35 |  |  | Same as 8.16.35 |  |  |  |  |  |  | Same as 8.16.35 |  |
| 8.16.37 3DL FDD CA Activation and Deactivation of known SCell in non-DRX |  |  | Same as 8.16.35 |  |  | Same as 8.16.35 |  |  |  |  |  |  | Same as 8.16.35 |  |
| 8.16.38 3DL TDD CA Activation and Deactivation of known SCell in non-DRX |  |  | Same as 8.16.35 |  |  | Same as 8.16.35 |  |  |  |  |  |  | Same as 8.16.35 |  |
| 8.16.39 E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -infinity E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB During T2, T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2, T3: 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -infinity E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB During T2, T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB |  |
| 8.16.40 E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD |  |  | Same as 8.16.39 |  |  |  |  | Same as 8.16.39 |  |  |  |  | Same as 8.16.39 |  |
| 8.16.41 3 DL FDD CA activation and deactivation of unknown SCell in non-DRX |  |  | Same as 8.16.39 |  |  |  |  | Same as 8.16.39 |  |  |  |  | Same as 8.16.39 |  |
| 8.16.42 3 DL TDD CA activation and deactivation of unknown SCell in non-DRX |  |  | Same as 8.16.39 |  |  |  |  | Same as 8.16.39 |  |  |  |  | Same as 8.16.39 |  |
| 8.16.47 2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX |  |  | Same as 8.16.17 |  |  |  |  | Same as 8.16.17 |  |  |  |  | Same as 8.16.17 |  |
| 8.16.48 2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX |  |  | Same as 8.16.17 |  |  |  |  | Same as 8.16.17 |  |  |  |  | Same as 8.16.17 |  |
| 8.16.49 2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX |  |  | Same as 8.16.17 |  |  |  |  | Same as 8.16.17 |  |  |  |  | Same as 8.16.17 |  |
| 8.16.50 2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX |  |  | Same as 8.16.17 |  |  |  |  | Same as 8.16.17 |  |  |  |  | Same as 8.16.17 |  |
| 8.16.51 E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -infinity E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB E-UTRA Cell 5 Ês5 / Noc4: -infinity During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB E-UTRA Cell 5 Ês5 / Noc4: 17dB During T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: -3dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -3dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: -3dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: -3dB E-UTRA Cell 5 Ês5 / Noc4: -3dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB +0.2dB -0.5dB +0.7dB 0dB During T2: 0dB 0dB 0dB +0.2dB 0dB +0.2dB -0.5dB +0.7dB 0dB During T3: 0dB 0dB 0dB -0.2dB 0dB -0.2dB -0.5dB -0.7dB -0.7dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -infinity E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17.2dB E-UTRA Cell 4 Noc4: -104.5dBm/15kHz Ês4 / Noc4: 17.7dB E-UTRA Cell 5 Ês5 / Noc4: -infinity During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17.2dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17.2dB E-UTRA Cell 4 Noc4: -104.5dBm/15kHz Ês4 / Noc4: 17.7dB E-UTRA Cell 5 Ês5 / Noc4: 17dB During T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: -3dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -3.2dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: -3.2dB E-UTRA Cell 4 Noc4: -104.5dBm/15kHz Ês4 / Noc4: -3.7dB E-UTRA Cell 5 Ês5 / Noc4: -3.7dB |  |
| 8.16.52 E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX |  |  | Same as 8.16.51 |  |  |  |  | Same as 8.16.51 |  |  |  |  |  |  |
| 8.16.53 4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX |  |  | Same as 8.16.51 |  |  |  |  | Same as 8.16.51 |  |  |  |  |  |  |
| 8.16.54 4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX |  |  | Same as 8.16.51 |  |  |  |  | Same as 8.16.51 |  |  |  |  |  |  |
| 8.16.55 4DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX |  |  | During T1: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Noc4: -101dBm/15kHz Ês4 / Noc4: 16dB E-UTRA Cell 5 Ês5 / Noc4: -infinity During T2: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Noc4: -101dBm/15kHz Ês4 / Noc4: 16dB E-UTRA Cell 5 Ês5 / Noc4: 16dB During T3: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3:16dB E-UTRA Cell 4 Noc4: -101dBm/15kHz Ês4 / Noc4:16dB E-UTRA Cell 5 Ês5 / Noc4: -infinity During T4: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Noc4: -101dBm/15kHz Ês4 / Noc4: 16dB E-UTRA Cell 5 Ês5 / Noc4: 16dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16 E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Noc4: -101dBm/15kHz Ês4 / Noc4: 16dB E-UTRA Cell 5 Ês5 / Noc4: -infinity During T2: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Noc4: -101dBm/15kHz Ês4 / Noc4: 16dB E-UTRA Cell 5 Ês5 / Noc4: 16dB During T3: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Noc4: -101dBm/15kHz Ês4 / Noc4: 16dB E-UTRA Cell 5 Ês5 / Noc4: -infinity E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Noc4: -101dBm/15kHz Ês4 / Noc4: 16dB E-UTRA Cell 5 Ês5 / Noc4: 16dB |  |
| 8.16.56 4DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX |  |  | Same as 8.16.55 |  |  |  |  | Same as 8.16.55 |  |  |  |  | Same as 8.16.55 |  |
| 8.16.57 4DL FDD CA Activation and Deactivation of Known SCell in Non-DRX |  |  | During T1, T2, T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB |  |  |  |  | During T1,T2,T3: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1, T2, T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB |  |
| 8.16.58 4DL TDD CA Activation and Deactivation of Known SCell in Non-DRX |  |  | Same as 8.16.57 |  |  |  |  | Same as 8.16.57 |  |  |  |  | Same as 8.16.57 |  |
| 8.16.59 4DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX |  |  | Same as 8.16.57 |  |  |  |  | Same as 8.16.57 |  |  |  |  | Same as 8.16.57 |  |
| 8.16.60 4DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX |  |  | Same as 8.16.57 |  |  |  |  | Same as 8.16.57 |  |  |  |  | Same as 8.16.57 |  |
| 8.16.61 4DL FDD CA Activation and Deactivation of Unknown SCell in Non-DRX |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -infinity E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB During T2, T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T2, T3: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -infinity E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB During T2, T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB |  |
| 8.16.62 4DL TDD CA Activation and Deactivation of Unknown SCell in Non-DRX |  |  | Same as 8.16.61 |  |  |  |  | Same as 8.16.61 |  |  |  |  | Same as 8.16.61 |  |
| 8.16.63 4DL PCell in FDD CA Activation and Deactivation of Unknown SCell in Non-DRX |  |  | Same as 8.16.61 |  |  |  |  | Same as 8.16.61 |  |  |  |  | Same as 8.16.61 |  |
| 8.16.64 4DL PCell in TDD CA Activation and Deactivation of Unknown SCell in Non-DRX |  |  | Same as 8.16.61 |  |  |  |  | Same as 8.16.61 |  |  |  |  | Same as 8.16.61 |  |
| 8.16.65 5 DL PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -infinity E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB E-UTRA Cell 5 Noc5: -104dBm/15kHz Ês5 / Noc5: 17dB E-UTRA Cell 6 Ês6 / Noc5: -infinity During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB E-UTRA Cell 5 Noc5: -104dBm/15kHz Ês5 / Noc5: 17dB E-UTRA Cell 6 Ês6 / Noc5: 17dB During T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: -3dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -3dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: -3dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: -3dB E-UTRA Cell 5 Noc5: -104dBm/15kHz Ês5 / Noc5: -3dB E-UTRA Cell 6 Ês6 / Noc5: -3dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB +0.2dB 0dB +0.2dB -0.5dB +0.7dB 0dB During T2: 0dB 0dB 0dB +0.2dB 0dB +0.2dB 0dB +0.2dB -0.5dB +0.7dB 0dB During T3: 0dB 0dB 0dB -0.2dB 0dB -0.2dB 0dB -0.2dB -0.5dB -0.7dB -0.7dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -infinity E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17.2dB E-UTRA Cell 4 Noc4: -104.5dBm/15kHz Ês4 / Noc4: 17.7dB E-UTRA Cell 5 Noc5: -104.5dBm/15kHz Ês5 / Noc5: 17.7dB E-UTRA Cell 6 Ês6 / Noc5: -infinity During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17.2dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17.2dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17.2dB E-UTRA Cell 5 Noc5: -104.5dBm/15kHz Ês5 / Noc5: 17.7dB E-UTRA Cell 6 Ês6 / Noc5: 17dB During T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: -3dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -3.2dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: -3.2dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: -3.2dB E-UTRA Cell 5 Noc5: -104.5dBm/15kHz Ês5 / Noc5: -3.7dB E-UTRA Cell 6 Ês6 / Noc5: -3.7dB |  |
| 8.16.66 5 DL PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX |  |  | Same as 8.16.65 |  |  |  |  | Same as 8.16.65 |  |  |  |  | Same as 8.16.65 |  |
| 8.16.67 5DL Pcell in FDD CA activation and deactivation of unknown SCell in non-DRX |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -infinity E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB E-UTRA Cell 5 Noc5: -104dBm/15kHz Ês5 / Noc5: 17dB During T2, T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB E-UTRA Cell 5 Noc5: -104dBm/15kHz Ês5 / Noc5: 17dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T2, T3: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -infinity E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB Noc5: -104dBm/15kHz Ês5 / Noc5: 17dB During T2, T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB E-UTRA Cell 5 Noc5: -104dBm/15kHz Ês5 / Noc5: 17dB |  |
| 8.16.68 5DL Pcell in TDD CA activation and deactivation of unknown SCell in non-DRX |  |  | Same as 8.16.67 |  |  |  |  | Same as 8.16.67 |  |  |  |  | Same as 8.16.67 |  |
| 8.16.69 5DL FDD CA activation and deactivation of unknown SCell in non-DRX |  |  | Same as 8.16.67 |  |  |  |  | Same as 8.16.67 |  |  |  |  | Same as 8.16.67 |  |
| 8.16.70 5DL TDD CA activation and deactivation of unknown SCell in non-DRX |  |  | Same as 8.16.67 |  |  |  |  | Same as 8.16.67 |  |  |  |  | Same as 8.16.67 |  |
| 8.16.71 5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX |  |  | Same as 8.16.65 |  |  |  |  | Same as 8.16.65 |  |  |  |  | Same as 8.16.65 |  |
| 8.16.72 5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX |  |  | Same as 8.16.65 |  |  |  |  | Same as 8.16.65 |  |  |  |  | Same as 8.16.65 |  |
| 8.16.73 5DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX |  |  | During T1, T2, T3 and T4: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Noc4: -101dBm/15kHz Ês4 / Noc4: 16dB E-UTRA Cell 5 Noc5: -101dBm/15kHz Ês5 / Noc5: 16dB During T1 and T3: E-UTRA Cell 6 Ês6 / Noc5: -infinity During T2 and T4: E-UTRA Cell 6 Ês6 / Noc5: 16dB |  |  |  |  | During T1, T2, T3 and T4: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T1 and T3: 0dB During T2 and T2: 0dB |  |  |  |  | During T1, T2, T3 and T4: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Noc4: -101dBm/15kHz Ês4 / Noc4: 16dB E-UTRA Cell 5 Noc5: -101dBm/15kHz Ês5 / Noc5: 16dB During T1 and T3: E-UTRA Cell 6 Ês6 / Noc5: -infinity During T2 and T4: E-UTRA Cell 6 Ês6 / Noc5: 16dB |  |
| 8.16.74 5DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX |  |  | Same as 8.16.73 |  |  |  |  | Same as 8.16.73 |  |  |  |  | Same as 8.16.73 |  |
| 8.16.75 5DL PCell in FDD CA activation and deactivation of known SCell in non-DRX |  |  | During T1, T2, T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB E-UTRA Cell 5 Noc5: -104dBm/15kHz Ês5 / Noc5: 17dB |  |  |  |  | During T1,T2,T3: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1, T2, T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB E-UTRA Cell 5 Noc5: -104dBm/15kHz Ês5 / Noc5: 17dB |  |
| 8.16.76 5DL PCell in TDD CA activation and deactivation of known SCell in non-DRX |  |  | Same as 8.16.75 |  |  |  |  | Same as 8.16.75 |  |  |  |  | Same as 8.16.75 |  |
| 8.16.77 5DL FDD CA activation and deactivation of known SCell in non-DRX |  |  | Same as 8.16.75 |  |  |  |  | Same as 8.16.75 |  |  |  |  | Same as 8.16.75 |  |
| 8.16.78 E-UTRAN TDD 5DL CA activation and deactivation of known SCell in non-DRX |  |  | Same as 8.16.75 |  |  |  |  | Same as 8.16.75 |  |  |  |  | Same as 8.16.75 |  |
| 8.16.83 3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes |  |  | Same as 8.16.27 |  |  |  |  | Same as 8.16.27 |  |  |  |  | Same as 8.16.27 The Test Tolerance values are taken from the analysis for the related 3DL Test case 8.16.27, and no separate analysis is needed. |  |
| 8.16.84 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes |  |  | Same as 8.16.31 |  |  |  |  | Same as 8.16.31 |  |  |  |  | Same as 8.16.31 The Test Tolerance values are taken from the analysis for the related 3DL Test case 8.16.31, and no separate analysis is needed. |  |
| 8.16.85 3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes |  |  | Same as 8.16.35 |  |  |  |  | Same as 8.16.35 |  |  |  |  | Same as 8.16.35 The Test Tolerance values are taken from the analysis for the related 3DL Test case 8.16.35, and no separate analysis is needed. |  |
| 8.16.86 3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes |  |  | Same as 8.16.39 |  |  |  |  | Same as 8.16.39 |  |  |  |  | Same as 8.16.39 The Test Tolerance values are taken from the analysis for the related 3DL Test case 8.16.39, and no separate analysis is needed. |  |
| 8.16.87 4 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes |  |  | Same as 8.16.51 |  |  |  |  | Same as 8.16.51 |  |  |  |  | Same as 8.16.51 |  |
| 8.16.88 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes |  |  | Same as 8.16.55 |  |  |  |  | Same as 8.16.55 |  |  |  |  | Same as 8.16.55 |  |
| 8.16.91 5 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes |  |  | Same as 8.16.65 |  |  |  |  | Same as 8.16.65 |  |  |  |  | Same as 8.16.65 |  |
| 8.16.92 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes |  |  | Same as 8.16.73 |  |  |  |  | Same as 8.16.73 |  |  |  |  | Same as 8.16.73 |  |
| 8.16.89 4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes |  |  | Same as 8.16.57 |  |  |  |  | Same as 8.16.57 |  |  |  |  | Same as 8.16.57 The Test Tolerance values are taken from the analysis for the related 3DL Test case 8.16.35, and no separate analysis is needed. |  |
| 8.16.90 4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes |  |  | Same as 8.16.61 |  |  |  |  | Same as 8.16.61 |  |  |  |  | Same as 8.16.61 The Test Tolerance values are taken from the analysis for the related 3DL Test case 8.16.39, and no separate analysis is needed. |  |
| 8.16.93 5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes |  |  | Same as 8.16.75 |  |  |  |  | Same as 8.16.75 |  |  |  |  | Same as 8.16.75 The Test Tolerance values are taken from the analysis for the related 3DL Test case 8.16.35, and no separate analysis is needed. |  |
| 8.16.94 5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes |  |  | Same as 8.16.67 |  |  |  |  | Same as 8.16.67 |  |  |  |  | Same as 8.16.67 The Test Tolerance values are taken from the analysis for the related 3DL Test case 8.16.39, and no separate analysis is needed. |  |
| 8.16.95 6DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes |  |  | Same as 8.16.71 |  |  |  |  | Same as 8.16.71 |  |  |  |  | Same as 8.16.71 The Test Tolerance values are taken from the analysis for the related 5DL Test case 8.16.71, and no separate analysis is needed. |  |
| 8.16.96 6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes |  |  | Same as 8.16.73 |  |  |  |  | Same as 8.16.73 |  |  |  |  | Same as 8.16.73 The Test Tolerance values are taken from the analysis for the related 5DL Test case 8.16.73, and no separate analysis is needed. |  |
| 8.16.97 6 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes |  |  | Same as 8.16.67 |  |  |  |  | Same as 8.16.67 |  |  |  |  | Same as 8.16.67 The Test Tolerance values are taken from the analysis for the related 3DL Test case 8.16.39, and no separate analysis is needed. |  |
| 8.16.98 6 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes |  |  | Same as 8.16.67 |  |  |  |  | Same as 8.16.67 |  |  |  |  | Same as 8.16.67 The Test Tolerance values are taken from the analysis for the related 3DL Test case 8.16.39, and no separate analysis is needed. |  |
| 8.16.99 7DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes |  |  | Same as 8.16.71 |  |  |  |  | Same as 8.16.71 |  |  |  |  | Same as 8.16.71 The Test Tolerance values are taken from the analysis for the related 5DL Test case 8.16.71, and no separate analysis is needed. |  |
| 8.16.100 7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes |  |  | Same as 8.16.73 |  |  |  |  | Same as 8.16.73 |  |  |  |  | Same as 8.16.73 The Test Tolerance values are taken from the analysis for the related 5DL Test case 8.16.73, and no separate analysis is needed. |  |
| 8.16.101 7 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes |  |  | Same as 8.16.67 |  |  |  |  | Same as 8.16.67 |  |  |  |  | Same as 8.16.67 The Test Tolerance values are taken from the analysis for the related 3DL Test case 8.16.39, and no separate analysis is needed. |  |
| 8.16.102 7 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes |  |  | Same as 8.16.67 |  |  |  |  | Same as 8.16.67 |  |  |  |  | Same as 8.16.67 The Test Tolerance values are taken from the analysis for the related 3DL Test case 8.16.39, and no separate analysis is needed. |  |
| 8.20.1 UE Measurement Procedures / RAT Measurements in CA mode / E-UTRAN FDD-FDD inter frequency event triggered reporting under fading propagation conditions in asynchronous cells |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: -infinity E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: 4dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: 7dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: 4dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0.7dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: -infinity E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: 4dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: 7.7dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: 4dB |  |
| 8.20.2 UE Measurement Procedures / RAT Measurements in CA mode / E-UTRAN TDD-TDD inter frequency event triggered reporting under fading propagation conditions in asynchronous cells |  |  | Same as 8.20.1 |  |  |  |  | Same as 8.20.1 |  |  |  |  | Same as 8.20.1 |  |
| 8.20.2A UE Measurement Procedures / RAT Measurements in CA mode / E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth |  |  | Same as 8.20.1 |  |  |  |  | Same as 8.20.1 |  |  |  |  | Same as 8.20.1 |  |
| 8.20.2B UE Measurement Procedures / RAT Measurements in CA mode / E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth |  |  | Same as 8.20.1 |  |  |  |  | Same as 8.20.1 |  |  |  |  | Same as 8.20.1 |  |
| 8.20.3 E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -1.8 dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -1.8 dB |  |
| 8.20.4 E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +9.00dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: +9.00dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: -infinity PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +9.00dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: +9.00dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: 5.00dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +9.00dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: +9.00dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: -infinity PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +9.00dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: +9.00dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: 5.00dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |
| 8.20.4A E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions for 20 MHz + 20 MHz bandwidth |  |  | Same as 8.20.4 |  |  |  |  | Same as 8.20.4 |  |  |  |  | Same as 8.20.4 |  |
| 8.20.4B E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions for 20 MHz + 10 MHz bandwidth |  |  | Same as 8.20.4 |  |  |  |  | Same as 8.20.4 |  |  |  |  | Same as 8.20.4 |  |
| 8.22.1 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal |  |  | Same as 8.1.1 |  |  |  |  | Same as 8.1.1 |  |  |  |  | Same as 8.1.1 |  |
| 8.22.2 E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal |  |  | Same as 8.1.1 |  |  |  |  | Same as 8.1.1 |  |  |  |  | Same as 8.1.1 |  |
| 8.22.3 E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB |  |
| 8.22.4 E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal |  |  | Same as 8.22.3 |  |  |  |  | Same as 8.22.3 |  |  |  |  | Same as 8.22.3 |  |
| 8.22.5 E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal |  |  | During T1: Noc: -98dBm/15kHz CRS Ês1 / Noc: +4.00dB CRS Ês2 / Noc: -infinity CSI-RS Ês1 / Noc: +10.00dB CSI-RS Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB CSI-RS Ês1 / Noc: +10.00dB CSI-RS Ês2 / Noc: +10.00dB |  |  |  |  | During T1: 0dB 2.10dB 0dB 2.10dB 2.10dB During T2: 0dB 2.10dB 2.10dB 2.10dB 2.10dB |  |  |  |  | During T1: Noc: -98dBm/15kHz CRS Ês1 / Noc: +6.10dB CRS Ês2 / Noc: -infinity CSI-RS Ês1 / Noc: +12.10dB CSI-RS Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +6.10dB Ês2 / Noc: +6.10dB CSI-RS Ês1 / Noc: +12.10dB CSI-RS Ês2 / Noc: +12.10dB |  |
| 8.22.6 E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity CSI-RS Ês1 / Noc: +10.00dB CSI-RS Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB CSI-RS Ês1 / Noc: +10.00dB CSI-RS Ês2 / Noc: +10.00dB |  |  |  |  | During T1: 0dB 2.60dB 0dB 2.60dB 0dB During T2: 0dB 2.60dB 2.60dB 2.60dB 2.60dB |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +6.60dB Ês2 / Noc: -infinity CSI-RS Ês1 / Noc: +12.60dB CSI-RS Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +6.60dB Ês2 / Noc: +6.60dB CSI-RS Ês1 / Noc: +12.60dB CSI-RS Ês2 / Noc: +12.60dB |  |
| 8.22.7 E-UTRAN FDD-FDD inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHzCRS Ês1 / Noc1: +4.00dB CRS Ês2 / Noc2: -infinity CSI-RS Ês1 / Noc1: +10.00dB CSI-RS Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: +7.00dB CSI-RS Ês1 / Noc1: +10.00dB CSI-RS Ês2 / Noc2: +13.00dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0.2dB 0dB 0.2dB |  |  |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHzCRS Ês1 / Noc1: +4dB CRS Ês2 / Noc2: -infinity CSI-RS Ês1 / Noc1: +10.00dB CSI-RS Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +7.2dB CSI-RS Ês1 / Noc1: +10.00dB CSI-RS Ês2 / Noc2: +13.20dB |  |
| 8.22.8 E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHzCRS Ês1 / Noc1: +4.00dB CRS Ês2 / Noc2: -infinity CSI-RS Ês1 / Noc1: +10.00dB CSI-RS Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: +7.00dB CSI-RS Ês1 / Noc1: +10.00dB CSI-RS Ês2 / Noc2: +13.00dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHzCRS Ês1 / Noc1: +4dB CRS Ês2 / Noc2: -infinity CSI-RS Ês1 / Noc1: +10.00dB CSI-RS Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +7.2dB CSI-RS Ês1 / Noc1: +10.00dB CSI-RS Ês2 / Noc2: +13.20dB |  |
| 8.22.9 E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal |  |  | Same as 8.16.1 |  |  |  |  | Same as 8.16.1 |  |  |  |  | Same as 8.16.1 |  |
| 8.22.10 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal |  |  | Same as 8.16.1 |  |  |  |  | Same as 8.16.1 |  |  |  |  | Same as 8.16.1 |  |
| 8.22.11 E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal |  |  | During T1: E-UTRA Cell 1 Noc1: -101dBm/15kHz CRS Ês1 / Noc1: -3dB CSI-RS Ês1 / Noc1: 3dB E-UTRA Cell 2 Noc2: -101dBm/15kHz CRS Ês2 / Noc2: -3dB CSI-RS Ês2 / Noc2: 3dB E-UTRA Cell 3 CRS Ês3 / Noc2: -infinity CSI-RS Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -101dBm/15kHz CRS Ês1 / Noc1: 17dB CSI-RS Ês1 / Noc1: 23dB E-UTRA Cell 2 Noc2: -101dBm/15kHz CRS Ês2 / Noc2: 17dB CSI-RS Ês2 / Noc2: 23dB E-UTRA Cell 3 CRS Ês3 / Noc2: -infinity CSI-RS Ês3 / Noc2: -infinity During T3: E-UTRA Cell 1 Noc1: -101dBm/15kHz CRS Ês1 / Noc1: 17dB CSI-RS Ês1 / Noc1: 23dB E-UTRA Cell 2 Noc2: -101dBm/15kHz CRS Ês2 / Noc2: 17dB CSI-RS Ês2 / Noc2: 23dB E-UTRA Cell 3 CRS Ês3 / Noc2: 17dB CSI-RS Ês3 / Noc2: 23dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -101dBm/15kHz CRS Ês1 / Noc1: -3dB CSI-RS Ês1 / Noc1: 3dB E-UTRA Cell 2 Noc2: -101dBm/15kHz CRS Ês2 / Noc2: -3dB CSI-RS Ês2 / Noc2: 3dB E-UTRA Cell 3 CRS Ês3 / Noc2: -infinity CSI-RS Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -101dBm/15kHz CRS Ês1 / Noc1: 17dB CSI-RS Ês1 / Noc1: 23dB E-UTRA Cell 2 Noc2: -101dBm/15kHz CRS Ês2 / Noc2: 17dB CSI-RS Ês2 / Noc2: 23dB E-UTRA Cell 3 CRS Ês3 / Noc2: -infinity CSI-RS Ês3 / Noc2: -infinity During T3: E-UTRA Cell 1 Noc1: -101dBm/15kHz CRS Ês1 / Noc1: 17dB CSI-RS Ês1 / Noc1: 23dB E-UTRA Cell 2 Noc2: -101dBm/15kHz CRS Ês2 / Noc2: 17dB CSI-RS Ês2 / Noc2: 23dB E-UTRA Cell 3 CRS Ês3 / Noc2: 17dB CSI-RS Ês3 / Noc2: 23dB |  |
| 8.22.12 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal |  |  | Same as 8.22.11 |  |  |  |  | Same as 8.22.11 |  |  |  |  | Same as 8.22.11 |  |
| 8.23.1 E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC |  |  | During T1: E-UTRA Cell 1 Noc: -104dBm/15kHz Ês / Noc: 16dB E-UTRA Cell 2 Noc: -104dBm/15kHz Ês / Noc: 16dB During T2: E-UTRA Cell 1 Noc: -104dBm/15kHz Ês / Noc: -2.5dB E-UTRA Cell 2 Noc: -104dBm/15kHz Ês / Noc: 2.5dB During T3: E-UTRA Cell 1 Noc: -104dBm/15kHz Ês / Noc: 20dB E-UTRA Cell 2 Noc: -104dBm/15kHz Ês / Noc: 20dB |  |  |  |  | During T1: 0dB 0.2dB 0dB 0.2dB During T2: 0dB -0.2dB 0dB -0.2dB During T3: 0dB 0.2dB 0dB 0.2dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -104dBm/15kHz Ês / Noc: 16.2dB E-UTRA Cell 2 Noc: -104dBm/15kHz Ês / Noc: 16.2dB During T2: E-UTRA Cell 1 Noc: -104dBm/15kHz Ês / Noc: -2.7dB E-UTRA Cell 2 Noc: -104dBm/15kHz Ês / Noc: -2.7dB During T3: E-UTRA Cell 1 Noc: -104dBm/15kHz Ês / Noc: 20.2dB E-UTRA Cell 2 Noc: -104dBm/15kHz Ês / Noc: 20.2dB |  |
| 8.23.2 E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC |  |  | Same as 8.23.1 |  |  |  |  | Same as 8.23.1 |  |  |  |  | Same as 8.23.1 |  |
| 8.23.3 E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC |  |  | Same as 8.23.1 |  |  |  |  | Same as 8.23.1 |  |  |  |  | Same as 8.23.1 |  |
| 8.23.4 E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC |  |  | During T1: E-UTRA Cell 1 Noc: -101dBm/15kHz Ês / Noc: 4dB E-UTRA Cell 2 Noc: -101dBm/15kHz Ês / Noc: 4dB E-UTRA Cell 3 Noc: -101dBm/15kHz Ês / Noc: -infinity During T2: E-UTRA Cell 1 Noc: -101dBm/15kHz Ês / Noc: 4dB E-UTRA Cell 2 Noc: -101dBm/15kHz Ês / Noc: 4dB E-UTRA Cell 3 Noc: -101dBm/15kHz Ês / Noc: 7dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -101dBm/15kHz Ês / Noc: 4dB E-UTRA Cell 2 Noc: -101dBm/15kHz Ês / Noc: 4dB E-UTRA Cell 3 Noc: -101dBm/15kHz Ês / Noc: -infinity During T2: E-UTRA Cell 1 Noc: -101dBm/15kHz Ês / Noc: 4dB E-UTRA Cell 2 Noc: -101dBm/15kHz Ês / Noc: 4dB E-UTRA Cell 3 Noc: -101dBm/15kHz Ês / Noc: 7dB |  |
| 8.23.5 E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC |  |  | Same as 8.23.4 |  |  |  |  | Same as 8.23.4 |  |  |  |  | Same as 8.23.4 |  |
| 8.23.6 E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC |  |  | Same as 8.23.4 |  |  |  |  | Same as 8.23.4 |  |  |  |  | Same as 8.23.4 |  |
| 8.26.1 E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB |  |
| 8.26.2 E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX |  |  | Same as 8.26.1 |  |  |  |  | Same as 8.26.1 |  |  |  |  | Same as 8.26.1 |  |
| 8.26.3 E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX |  |  | During T1: E-UTRA Cell 1 Noc1: -98dBm/15kHz Ês1 / Noc1: 13dB E-UTRA Cell 2 Noc2: -98dBm/15kHz Ês2 / Noc2: 13dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -98dBm/15kHz Ês1 / Noc1: 13dB E-UTRA Cell 2 Noc2: -98dBm/15kHz Ês2 / Noc2: 13dB E-UTRA Cell 3 Ês3 / Noc2: 13dB |  |  |  |  | During T1: 0dB 0dB 0dB -3.1dB 0dB During T2: 0dB 0dB 0dB -3.1dB -3.1dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -98dBm/15kHz Ês1 / Noc1: 13dB E-UTRA Cell 2 Noc2: -98dBm/15kHz Ês2 / Noc2: 9.9dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -98dBm/15kHz Ês1 / Noc1: 13dB E-UTRA Cell 2 Noc2: -98dBm/15kHz Ês2 / Noc2: 9.9dB E-UTRA Cell 3 Ês3 / Noc2: 9.9dB |  |
| 8.26.3A E-UTRAN FDD-FS3 3-DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX |  |  | During T1: E-UTRA Cell 1 Noc1: -98dBm/15kHz Ês1 / Noc1: 13dB E-UTRA Cell 2 Noc2: -98dBm/15kHz Ês2 / Noc2: 13dB E-UTRA Cell 3 Noc2: -98dBm/15kHz Ês3 / Noc2: 13dB E-UTRA Cell 4 Ês4 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -98dBm/15kHz Ês1 / Noc1: 13dB E-UTRA Cell 2 Noc2: -98dBm/15kHz Ês2 / Noc2: 13dB E-UTRA Cell 3 Noc2: -98dBm/15kHz Ês3 / Noc2: 13dB E-UTRA Cell 4 Ês4 / Noc2: 13dB |  |  |  |  | During T1: 0dB 0dB 0dB -3.1dB 0dB -3.1dB 0dB During T2: 0dB 0dB 0dB -3.1dB 0dB -3.1dB -3.1dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -98dBm/15kHz Ês1 / Noc1: 13dB E-UTRA Cell 2 Noc2: -98dBm/15kHz Ês2 / Noc2: 9.9dB E-UTRA Cell 3 Noc2: -98dBm/15kHz Ês3 / Noc2: 9.9dB E-UTRA Cell 4 Ês4 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -98dBm/15kHz Ês1 / Noc1: 13dB E-UTRA Cell 2 Noc2: -98dBm/15kHz Ês2 / Noc2: 9.9dB E-UTRA Cell 3 Noc2: -98dBm/15kHz Ês3 / Noc2: 9.9dB E-UTRA Cell 4 Ês4 / Noc2: 9.9dB |  |
| 8.26.4 E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX |  |  | Same as 8.26.3 |  |  |  |  | Same as 8.26.3 |  |  |  |  | Same as 8.26.3 |  |
| 8.26.5 E-UTRAN FDD-TDD intra-frequency event triggered reporting in non-DRX for CRS based discovery signal under Operation with Frame Structure 3 |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 4dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 4dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 4dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 4dB E-UTRA Cell 3 Ês3 / Noc2: 4dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 4dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 4dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 4dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 4dB E-UTRA Cell 3 Ês3 / Noc2: 4dB |  |
| 8.26.6 E-UTRAN TDD-TDD intra-frequency event triggered reporting in non-DRX for CRS based discovery signal under Operation with Frame Structure 3 |  |  | Same as 8.26.5 |  |  |  |  | Same as 8.26.5 |  |  |  |  | Same as 8.26.5 |  |
| 8.26.7 E-UTRAN FDD-FS3 intra-frequency event triggered reporting in DRX for CRS based discovery signal |  |  | Same as 8.26.5 |  |  |  |  | Same as 8.26.5 |  |  |  |  | Same as 8.26.5 |  |
| 8.26.8 E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal |  |  | Same as 8.26.5 |  |  |  |  | Same as 8.26.5 |  |  |  |  | Same as 8.26.5 |  |
| 8.26.9 E-UTRAN FDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells |  |  | During T1: E-UTRA Cell 1 Noc1: -98dBm/15kHz Ês1 / Noc1: 4dB E-UTRA Cell 2 Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity dB During T2: E-UTRA Cell 1 Noc1: -98dBm/15kHz Ês1 / Noc1: 4dB E-UTRA Cell 2 Noc2: -98dBm/15kHz Ês2 / Noc2: 7dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -98dBm/15kHz Ês1 / Noc1: 4dB E-UTRA Cell 2 Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity dB During T2: E-UTRA Cell 1 Noc1: -98dBm/15kHz Ês1 / Noc1: 4dB E-UTRA Cell 2 Noc2: -98dBm/15kHz Ês2 / Noc2: 7dB |  |
| 8.26.10 E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells |  |  | Same as 8.26.9 |  |  |  |  | Same as 8.26.9 |  |  |  |  | Same as 8.26.9 |  |
| 9.1.1.1 FDD Intra Frequency Absolute RSRP Accuracy |  |  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±6dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported RSRP values: ±6dB |  |  |  |  | Test 1: -1.0dB 0dB +1.0dB Via mapping Test 2: 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +0.8dB Via mapping |  |  |  |  | Test 1: Noc: -107.0dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_29 to RSRP_43 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_45 to RSRP_64 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +3.0dB Ês2 / Noc: -0.2dB RSRP_17 to RSRP_32 RSRP_18 to RSRP_33 RSRP_19 to RSRP_34 RSRP_20 to RSRP_35 depending on operating band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. In all cases the RSRP values are 3dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.1.1_1 FDD Intra Frequency Absolute RSRP Accuracy (Rel 12 and forward) |  |  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported RSRP values: ±4.5dB |  |  |  |  | Test 1: -1.0dB 0dB +1.0dB Via mapping Test 2: 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +0.8dB Via mapping |  |  |  |  | Test 1: Noc: -107.0dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_29 to RSRP_42 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_45 to RSRP_64 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +3.0dB Ês2 / Noc: -0.2dB RSRP_18 to RSRP_30 RSRP_19 to RSRP_31 RSRP_20 to RSRP_32 RSRP_21 to RSRP_33 RSRP_22 to RSRP_34 depending on operating band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. The RSRP values are recalculated for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.1.1_2 FDD Intra Frequency Absolute RSRP Accuracy for UE category 1bis |  |  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±5.5dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±9dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported RSRP values: ±5.5dB |  |  |  |  | Test 1: -1.0dB 0dB +1.0dB Via mapping Test 2: 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +0.8dB Via mapping |  |  |  |  | Test 1: Noc: -107.0dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_28 to RSRP_43 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_44 to RSRP_65 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +3.0dB Ês2 / Noc: -0.2dB RSRP_17 to RSRP_31 RSRP_18 to RSRP_32 RSRP_19 to RSRP_33 RSRP_20 to RSRP_34 RSRP_21 to RSRP_35 depending on operating band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. The RSRP values are recalculated for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.1.1_3 FDD Intra Frequency Absolute RSRP Accuracy for CA Idle Mode Measurements |  |  | Test 1: Noc: -96dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +3.0dB Reported RSRP values: ±6dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +3.0dB Reported RSRP values: ±9.5dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +6.0dB Ês2 / Noc: +3.0dB Reported RSRP values: ±6dB |  |  |  |  | Test 1: -1.0dB 0dB +1.0dB Via mapping Test 2: 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +0.8dB Via mapping |  |  |  |  | Test 1: Noc: -97.0dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +4.0dB RSRP_42 to RSRP_54 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +4.0dB RSRP_48 to RSRP_67 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +6.0dB Ês2 / Noc: +3.8dB RSRP_23 to RSRP_35 RSRP_24 to RSRP_36 RSRP_24 to RSRP_36 RSRP_25 to RSRP_37 RSRP_25 to RSRP_37 RSRP_26 to RSRP_38 RSRP_27 to RSRP_39 depending on operating band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. The RSRP values are recalculated for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.1.2 FDD Intra Frequency Relative RSRP Accuracy |  |  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported relative RSRP values: ±3dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported relative RSRP values: ±3dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported relative RSRP values: ±3dB |  |  |  |  | Test 1: 0 dB 0 dB +1.0dB Via mapping Test 2: 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +1.0dB Via mapping |  |  |  |  | Test 1: Noc: -106 dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-9 to RSRP_x+1 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-9 to RSRP_x+1 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +3.0dB Ês2 / Noc: 0dB RSRP_x-8 to RSRP_x+2 |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc, Ês1 / Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for both normal and extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.1.2_2 FDD Intra Frequency Relative Accuracy of RSRP for UE category 1bis |  |  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported relative RSRP values: ±3dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported relative RSRP values: ±4dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported relative RSRP values: ±3dB |  |  | Test 1: 0 dB 0 dB +1.0dB Via mapping Test 2: 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +1.0dB Via mapping |  |  |  |  |  |  | Test 1: Noc: -106 dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-9 to RSRP_x+1 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-10 to RSRP_x+2 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +3.0dB Ês2 / Noc: 0dB RSRP_x-8 to RSRP_x+2 |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc, Ês1 / Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for both normal and extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.2.1 TDD Intra Frequency Absolute RSRP Accuracy |  |  | Same as 9.1.1.1 |  |  | Same as 9.1.1.1 |  |  |  |  |  |  | Same as 9.1.1.1 |  |
| 9.1.2.1_1 TDD Intra Frequency Absolute RSRP Accuracy (Rel 12 and forward) |  |  | Same as 9.1.1.1_1 |  |  | Same as 9.1.1.1_1 |  |  |  |  |  |  | Same as 9.1.1.1_1 |  |
| 9.1.2.1_2 TDD Intra Frequency Absolute RSRP Accuracy for UE category 1bis |  |  | Same as 9.1.1.1_2 |  |  | Same as 9.1.1.1_2 |  |  |  |  |  |  | Same as 9.1.1.1_2 |  |
| 9.1.2.2 TDD Intra Frequency Relative RSRP Accuracy |  |  | Same as 9.1.1.2 |  |  | Same as 9.1.1.2 |  |  |  |  |  |  | Same as 9.1.1.2 |  |
| 9.1.2.2_2 TDD Intra Frequency Relative Accuracy of RSRP for UE category 1bis |  |  | Same as 9.1.1.2_2 |  |  | Same as 9.1.1.2_2 |  |  |  |  |  |  | Same as 9.1.1.2_2 |  |
| 9.1.3.1 FDD Inter Frequency Absolute RSRP Accuracy |  |  | Test 1: Noc1: -88.65dBm/15kHz Ês1 / Noc1: +10.00dB Noc2: -88.65dBm/15kHz Ês2 / Noc2: +10.00dB Reported RSRP values: ±8dB Test 2: Noc1: (Noc2 +8dB) Ês1 / Noc1: +13.00dB Noc2: -117dBm to -113.5dBm /15kHz depending on operating band Ês2 / Noc2: -4.00dB Reported RSRP values: ±6dB |  |  | Test 1: -0.6dB 0dB -0.6dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  |  | Test 1: Noc1: -89.25dBm/15kHz Ês1 / Noc1: +10.00dB Noc2: -89.25dBm/15kHz Ês2 / Noc2: +10.00dB RSRP_52 to RSRP_71 Test 2: Noc1: (Noc2 +8dB) Ês1 / Noc1: +13.00dB Noc2: -117dBm to -113.5dBm /15kHz depending on operating band Ês2 / Noc: -3.20dB RSRP_13 to RSRP_28 RSRP_14 to RSRP_29 RSRP_15 to RSRP_30 RSRP_16 to RSRP_31 depending on operating band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. In all cases the RSRP values are 3dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.3.1_1 FDD Inter Frequency Absolute RSRP Accuracy (Rel 12 and forward) |  |  | Test 1: Noc1: -88.65dBm/15kHz Ês1 / Noc1: +10.00dB Noc2: -88.65dBm/15kHz Ês2 / Noc2: +10.00dB Reported RSRP values: ±8dB Test 2: Noc1: (Noc2 +8dB) Ês1 / Noc1: +13.00dB Noc2: -117dBm to -113.5dBm /15kHz depending on operating band Ês2 / Noc2: -4.00dB Reported RSRP values: ±6dB |  |  | Test 1: -0.6dB 0dB -0.6dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  |  | Test 1: Noc1: -89.25dBm/15kHz Ês1 / Noc1: +10.00dB Noc2: -89.25dBm/15kHz Ês2 / Noc2: +10.00dB RSRP_52 to RSRP_71 Test 2: Noc1: (Noc2 +8dB) Ês1 / Noc1: +13.00dB Noc2: -117dBm to -113.5dBm /15kHz depending on operating band Ês2 / Noc: -3.20dB Normal condition: RSRP_14 to RSRP_26 RSRP_15 to RSRP_27 RSRP_16 to RSRP_28 RSRP_17 to RSRP_29 RSRP_18 to RSRP_30 Extreme condition: RSRP_10 to RSRP_31 RSRP_11 to RSRP_32 RSRP_12 to RSRP_33 RSRP_13 to RSRP_34 depending on operating band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.3.1_2 FDD - FDD Inter Frequency Absolute RSRP Accuracy for UE category 1bis |  |  | Test 1: Noc1: -88.65dBm/15kHz Ês1 / Noc1: +10.00dB Noc2: -88.65dBm/15kHz Ês2 / Noc2: +10.00dB Reported RSRP values: ±9dB Test 2: Noc1: (Noc2 +8dB) Ês1 / Noc1: +13.00dB Noc2: -117dBm to -113.5dBm /15kHz depending on operating band Ês2 / Noc2: -4.00dB Reported RSRP values: ±5.5dB |  |  | Test 1: -0.6dB 0dB -0.6dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  |  | Test 1: Noc1: -89.25dBm/15kHz Ês1 / Noc1: +10.00dB Noc2: -89.25dBm/15kHz Ês2 / Noc2: +10.00dB RSRP_51 to RSRP_72 Test 2: Noc1: (Noc2 +8dB) Ês1 / Noc1: +13.00dB Noc2: -117dBm to -113.5dBm /15kHz depending on operating band Ês2 / Noc: -3.20dB Normal condition: RSRP_13 to RSRP_27 RSRP_14 to RSRP_28 RSRP_15 to RSRP_29 RSRP_16 to RSRP_30 RSRP_17 to RSRP_31 Extreme condition: RSRP_9 to RSRP_32 RSRP_10 to RSRP_33 RSRP_11 to RSRP_34 RSRP_12 to RSRP_35 depending on operating band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.3.1_3 FDD-FDD Inter Frequency Absolute RSRP Accuracy for CA Idle Mode Measurements for Overlapping Carrier |  |  | Test 1: Noc1: -88.65dBm/15kHz Ês1 / Noc1: +10.00dB Noc2: -88.65dBm/15kHz Ês2 / Noc2: +10.00dB Reported RSRP values: ±9dB Test 2: Noc1: (Noc2 +8dB) Ês1 / Noc1: +13.00dB Noc2: -117dBm to -113.5dBm /15kHz depending on operating band Ês2 / Noc2: -4.00dB Reported RSRP values: ±5.5dB |  |  | Test 1: -0.6dB 0dB -0.6dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  |  | Test 1: Noc1: -89.25dBm/15kHz Ês1 / Noc1: +10.00dB Noc2: -89.25dBm/15kHz Ês2 / Noc2: +10.00dB RSRP_51 to RSRP_72 Test 2: Noc1: (Noc2 +8dB) Ês1 / Noc1: +13.00dB Noc2: -117dBm to -113.5dBm /15kHz depending on operating band Ês2 / Noc: -3.20dB Normal condition: RSRP_13 to RSRP_27 RSRP_14 to RSRP_28 RSRP_15 to RSRP_29 RSRP_16 to RSRP_30 RSRP_17 to RSRP_31 Extreme condition: RSRP_9 to RSRP_32 RSRP_10 to RSRP_33 RSRP_11 to RSRP_34 RSRP_12 to RSRP_35 depending on operating band |  |
| 9.1.3.1_4 FDD-FDD Inter Frequency Absolute RSRP Accuracy for CA Idle Mode Measurements for Non-Overlapping Carrier |  |  | Test 1: Noc1: -95Bm/15kHz Ês1 / Noc1: +10.00dB Noc2: -95dBm/15kHz Ês2 / Noc2: +10.00dB Reported RSRP values: ±10.5dB Test 2: Noc1: (Noc2 +8dB) Ês1 / Noc1: +13.00dB Noc2: -117dBm to -113.5dBm /15kHz depending on operating band Ês2 / Noc2: -4.00dB Reported RSRP values: ±7dB |  |  | Test 1: -0.6dB 0dB -0.6dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  |  | Test 1: Noc1: -95.6dBm/15kHz Ês1 / Noc1: +10.00dB Noc2: -95.6dBm/15kHz Ês2 / Noc2: +10.00dB RSRP_45 to RSRP_66 Test 2: Noc1: (Noc2 +8dB) Ês1 / Noc1: +13.00dB Noc2: -117dBm to -113.5dBm /15kHz depending on operating band Ês2 / Noc: -3.20dB Normal condition: RSRP_14 to RSRP_28 RSRP_15 to RSRP_29 RSRP_15 to RSRP_29 RSRP_16 to RSRP_30 RSRP_16 to RSRP_30 RSRP_17 to RSRP_31 RSRP_18 to RSRP_32 depending on operating band |  |
| 9.1.3.2 FDD Inter Frequency Relative RSRP Accuracy |  |  | Test 1: Noc1: -88.65dBm/15kHz Noc2: -88.65dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +10dB Reported relative RSRP values: ±6dB Test 2: Noc1: (Noc2 +8dB) Noc2: -117dBm to -113.5dBm /15kHz depending on operating band Ês1 / Noc1: +13dB Ês2 / Noc2: -4.0dB Reported relative RSRP values: ±6dB |  |  | Test 1: -0.6dB -0.6dB 0dB 0dB Via mapping Test 2: -1.0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  |  | Test 1: Noc1: -89.25dBm/15kHz Noc2: -89.25dBm/15kHz Ês1 / Noc: +10dB Ês2 / Noc: +10dB RSRP_(x-9) to RSRP_(x+9) Test 2: Noc1: (Noc2 +7dB) Noc2: -117dBm to -113.5dBm /15kHz depending on operating band Ês1 / Noc1: +13dB Ês2 / Noc2: -3.2dB RSRP_(x-32) to RSRP_(x-16) |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc1 and Ês1 / Noc1 and Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for both normal and extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.3.2_1 FDD Inter Frequency Relative RSRP Accuracy (Rel 12 and forward) |  |  | Test 1: Noc1: -88.65dBm/15kHz Noc2: -88.65dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +10dB Reported relative RSRP values normal: ±4.5dB Reported relative RSRP values extreme : ±6dB Test 2: Noc1: (Noc2 +8dB) Noc2: -117dBm to -113.5dBm /15kHz depending on operating band Ês1 / Noc1: +13dB Ês2 / Noc2: -4.0dB Reported relative RSRP values normal: ±4.5dB Reported relative RSRP values extreme : ±6dB |  |  | Test 1: -0.6dB -0.6dB 0dB 0dB Via mapping Test 2: -1.0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  |  | Test 1: Noc1: -89.25dBm/15kHz Noc2: -89.25dBm/15kHz Ês1 / Noc: +10dB Ês2 / Noc: +10dB Normal condition: RSRP_(x-7) to RSRP_(x+7) Extreme condition: RSRP_(x-9) to RSRP_(x+9) Test 2: Noc1: (Noc2 +7dB) Noc2: -117dBm to -113.5dBm /15kHz depending on operating band Ês1 / Noc1: +13dB Ês2 / Noc2: -3.2dB Normal condition: RSRP_(x-30) to RSRP_(x-17) Extreme condition: RSRP_(x-32) to RSRP_(x-16) |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc1 and Ês1 / Noc1 and Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.3.2_2 FDD - FDD Inter Frequency Relative Accuracy of RSRP for UE category 1bis |  |  | Test 1: Noc1: -88.65dBm/15kHz Noc2: -88.65dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +10dB Reported relative RSRP values normal: ±5.5dB Reported relative RSRP values extreme : ±7dB Test 2: Noc1: (Noc2 +8dB) Noc2: -117dBm to -113.5dBm /15kHz depending on operating band Ês1 / Noc1: +13dB Ês2 / Noc2: -4.0dB Reported relative RSRP values normal: ±5.5dB Reported relative RSRP values extreme : ±7dB |  |  | Test 1: -0.6dB -0.6dB 0dB 0dB Via mapping Test 2: -1.0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  |  | Test 1: Noc1: -89.25dBm/15kHz Noc2: -89.25dBm/15kHz Ês1 / Noc: +10dB Ês2 / Noc: +10dB Normal condition: RSRP_(x-8) to RSRP_(x+8) Extreme condition: RSRP_(x-10) to RSRP_(x+10) Test 2: Noc1: (Noc2 +7dB) Noc2: -117dBm to -113.5dBm /15kHz depending on operating band Ês1 / Noc1: +13dB Ês2 / Noc2: -3.2dB Normal condition: RSRP_(x-31) to RSRP_(x-16) Extreme condition: RSRP_(x-33) to RSRP_(x-15) |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc1 and Ês1 / Noc1 and Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.4.1 TDD Inter Frequency Absolute RSRP Accuracy |  |  | Same as 9.1.3.1 |  |  | Same as 9.1.3.1 |  |  |  |  |  |  | Same as 9.1.3.1 |  |
| 9.1.4.1_1 TDD Inter Frequency Absolute RSRP Accuracy (R12 and forward) |  |  | Same as 9.1.3.1 |  |  | Same as 9.1.3.1 |  |  |  |  |  |  | Same as 9.1.3.1 |  |
| 9.1.4.1_2 TDD - TDD Inter Frequency Absolute RSRP Accuracy for UE category 1bis |  |  | Same as 9.1.3.1_2 |  |  | Same as 9.1.3.1_2 |  |  |  |  |  |  | Same as 9.1.3.1_2 |  |
| 9.1.4.2 TDD Inter Frequency Relative RSRP Accuracy |  |  | Same as 9.1.3.2 |  |  | Same as 9.1.3.2 |  |  |  |  |  |  | Same as 9.1.3.2 |  |
| 9.1.4.2_1 TDD Inter Frequency Relative RSRP Accuracy (Rel 12 and forward) |  |  | Same as 9.1.3.2 |  |  | Same as 9.1.3.2 |  |  |  |  |  |  | Same as 9.1.3.2 |  |
| 9.1.4.2_2 TDD - TDD Inter Frequency Relative Accuracy of RSRP for UE category 1bis |  |  | Same as 9.1.3.2_2 |  |  | Same as 9.1.3.2_2 |  |  |  |  |  |  | Same as 9.1.3.2_2 |  |
| 9.1.5.1 FDD-TDD inter frequency absolute RSRP Accuracy |  |  | Test 1: Noc1: -88.65dBm/15kHz Noc2: -88.65dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +10dB Reported absolute RSRP values: ±8dB Test 2: Noc1: -104dBm /15kHz Noc2: -112dBm/15kHz Ês1 / Noc1: +13dB Ês2 / Noc2: -4.0dB Reported absolute RSRP values: ±6dB |  |  | Test 1: -0.6dB -0.6dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  |  | Test 1: Noc1: -89.25dBm/15kHz Noc2: -89.25dBm/15kHz Ês1 / Noc: +10dB Ês2 / Noc: +10dB RSRP_52 to RSRP_71 Test 2: Noc1: -104Bm /15kHz Noc2: -112dBm /15kHz Ês1 / Noc1: +13dB Ês2 / Noc2: -3.2dB RSRP_18 to RSRP_33 |  |
| 9.1.5.1_1 FDD - TDD Inter Frequency Absolute RSRP Accuracy (Rel 12 and forward) |  |  | Test 1: Noc1: -88.65dBm/15kHz Noc2: -88.65dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +10dB Normal condition: Reported absolute RSRP values: ±8dB Extreme condition: Reported absolute RSRP values: ±11dB Test 2: Noc1: -104dBm /15kHz Noc2: -112dBm/15kHz Ês1 / Noc1: +13dB Ês2 / Noc2: -4.0dB Normal condition: Reported absolute RSRP values: ±4.5dB Extreme condition: Reported absolute RSRP values: ±8dB |  |  | Test 1: -0.6dB -0.6dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  |  | Test 1: Noc1: -89.25dBm/15kHz Noc2: -89.25dBm/15kHz Ês1 / Noc: +10dB Ês2 / Noc: +10dB RSRP_52 to RSRP_71 for normal condition RSRP_49 to RSRP_74 for extreme condition Test 2: Noc1: -104Bm /15kHz Noc2: -112dBm /15kHz Ês1 / Noc1: +13dB Ês2 / Noc2: -3.2dB RSRP_19 to RSRP_31 for normal condition RSRP_15 to RSRP_36 for extreme condition |  |
| 9.1.5.1_2 FDD - TDD Inter Frequency Absolute RSRP Accuracy for UE category 1bis |  |  | Same as 9.1.5.1_1 |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.5.1_2 FDD - TDD Inter Frequency Absolute RSRP Accuracy for UE category 1bis |  |  | Test 1: Noc1: -88.65dBm/15kHz Noc2: -88.65dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +10dB Normal condition: Reported absolute RSRP values: ±9dB Extreme condition: Reported absolute RSRP values: ±12dB Test 2: Noc1: -104dBm /15kHz Noc2: -112dBm/15kHz Ês1 / Noc1: +13dB Ês2 / Noc2: -4.0dB Normal condition: Reported absolute RSRP values: ±5.5dB Extreme condition: Reported absolute RSRP values: ±10dB |  |  | Test 1: -0.6dB -0.6dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  |  | Test 1: Noc1: -89.25dBm/15kHz Noc2: -89.25dBm/15kHz Ês1 / Noc: +10dB Ês2 / Noc: +10dB RSRP_51 to RSRP_72 for normal condition RSRP_48 to RSRP_75 for extreme condition Test 2: Noc1: -104Bm /15kHz Noc2: -112dBm /15kHz Ês1 / Noc1: +13dB Ês2 / Noc2: -3.2dB RSRP_18 to RSRP_32 for normal condition RSRP_14 to RSRP_37 for extreme condition |  |
| 9.1.5.2 FDD-TDD Inter Frequency Relative RSRP Accuracy |  |  | Test 1: Noc1: -88.65dBm/15kHz Noc2: -88.65dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +10dB Reported absolute RSRP values: ±6dB Test 2: Noc1: -104dBm /15kHz Noc2: -112dBm/15kHz Ês1 / Noc1: +13dB Ês2 / Noc2: -4.0dB Reported absolute RSRP values: ±6dB |  |  | Test 1: -0.6dB -0.6dB 0dB 0dB Via mapping Test 2: -1.0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  |  | Test 1: Noc1: -89.25dBm/15kHz Noc2: -89.25dBm/15kHz Ês1 / Noc: +10dB Ês2 / Noc: +10dB RSRP_(x-9) to RSRP_(x+9) Test 2: Noc1: -105.0Bm /15kHz Noc2: -112dBm /15kHz Ês1 / Noc1: +13dB Ês2 / Noc2: -3.2dB RSRP_(x-32) to RSRP_(x-16) |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc1 and Ês1 / Noc1 and Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for both normal and extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.5.2_1 FDD - TDD Inter Frequency Relative Accuracy of RSRP (Rel 12 and forward) |  |  | Test 1: Noc1: -88.65dBm/15kHz Noc2: -88.65dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +10dB Reported relative RSRP values normal: ±4.5dB Reported relative RSRP values extreme: ±6.0dB Test 2: Noc1: -104dBm /15kHz Noc2: -112dBm/15kHz Ês1 / Noc1: +13dB Ês2 / Noc2: -4.0dB Reported relative RSRP values normal: ±4.5dB Reported relative RSRP values extreme: ±6.0dB |  |  | Test 1: -0.6dB -0.6dB 0dB 0dB Via mapping Test 2: -1.0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  |  | Test 1: Noc1: -89.25dBm/15kHz Noc2: -89.25dBm/15kHz Ês1 / Noc: +10dB Ês2 / Noc: +10dB Normal condition: RSRP_(x-7) to RSRP_(x+7) Extreme condition: RSRP_(x-9) to RSRP_(x+9) Test 2: Noc1: -105.0Bm /15kHz Noc2: -112dBm /15kHz Ês1 / Noc1: +13dB Ês2 / Noc2: -3.2dB Normal condition: RSRP_(x-30) to RSRP_(x-17) Extreme condition: RSRP_(x-32) to RSRP_(x-16) |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc1 and Ês1 / Noc1 and Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.5.2_2 FDD - TDD Inter Frequency Relative Accuracy of RSRP for UE category 1bis |  |  | Test 1: Noc1: -88.65dBm/15kHz Noc2: -88.65dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +10dB Reported relative RSRP values normal: ±5.5dB Reported relative RSRP values extreme: ±7.0dB Test 2: Noc1: -104dBm /15kHz Noc2: -112dBm/15kHz Ês1 / Noc1: +13dB Ês2 / Noc2: -4.0dB Reported relative RSRP values normal: ±5.5dB Reported relative RSRP values extreme: ±7.0dB |  |  | Test 1: -0.6dB -0.6dB 0dB 0dB Via mapping Test 2: -1.0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  |  | Test 1: Noc1: -89.25dBm/15kHz Noc2: -89.25dBm/15kHz Ês1 / Noc: +10dB Ês2 / Noc: +10dB Normal condition: RSRP_(x-8) to RSRP_(x+8) Extreme condition: RSRP_(x-10) to RSRP_(x+10) Test 2: Noc1: -105.0Bm /15kHz Noc2: -112dBm /15kHz Ês1 / Noc1: +13dB Ês2 / Noc2: -3.2dB Normal condition: RSRP_(x-31) to RSRP_(x-16) Extreme condition: RSRP_(x-33) to RSRP_(x-15) |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc1 and Ês1 / Noc1 and Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.6.1 FDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / No2c: +3dB Ês3 / No2c: -1dB Reported RSRP values: ±6dB for normal conditions and ±9dB for extreme conditions |  |  | 0dB 0dB 0dB 0dB 0dB Via mapping Via mapping |  |  |  |  |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / No2c: +3dB Ês3 / No2c: -1dB Cell 1: RSRP_12 to RSRP_27 RSRP_13 to RSRP_28 RSRP_13 to RSRP_29 RSRP_14 to RSRP_29 RSRP_15 to RSRP_30 RSRP_15 to RSRP_31 depending on operating band Cell 2: RSRP_20 to RSRP_35 RSRP_21 to RSRP_36 RSRP_21 to RSRP_37 RSRP_22 to RSRP_37 RSRP_23 to RSRP_38 RSRP_23 to RSRP_39 depending on Cell 1 band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 RSRP from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRP from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. In all cases the RSRP values are 3dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.6.1_1 Measurement Performance Requirements / FDD Absolute RSRP Accuracy E-UTRA for Carrier Aggregation (Rel 12 and forward) |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Es1 / Noc1: -4dB Es2 / No2c: +3dB Es3 / No2c: -1dB Reported RSRP values: 4.5dB for normal conditions and 9dB for extreme conditions |  |  | 0dB 0dB 0dB 0dB 0dB Via mapping Via mapping |  |  |  |  |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Es1 / Noc1: -4dB Es2 / No2c: +3dB Es3 / No2c: -1dB Normal condition Cell 1: RSRP_13 to RSRP_26 RSRP_14 to RSRP_26 RSRP_14 to RSRP_27 RSRP_15 to RSRP_27 RSRP_15 to RSRP_28 RSRP_16 to RSRP_29 RSRP_17 to RSRP_29 depending on operating band Cell 2: RSRP_21 to RSRP_34 RSRP_22 to RSRP_34 RSRP_22 to RSRP_35 RSRP_23 to RSRP_35 RSRP_23 to RSRP_36 RSRP_24 to RSRP_37 RSRP_25 to RSRP_37 depending on Cell 1 band Exreme condition Cell 1: RSRP_9 to RSRP_30 RSRP_9 to RSRP_31 RSRP_10 to RSRP_31 RSRP_10 to RSRP_31 RSRP_11 to RSRP_32 RSRP_12 to RSRP_33 RSRP_12 to RSRP_34 depending on operating band Cell 2: RSRP_17 to RSRP_38 RSRP_17 to RSRP_39 RSRP_18 to RSRP_39 RSRP_18 to RSRP_40 RSRP_19 to RSRP_40 RSRP_20 to RSRP_41 RSRP_20 to RSRP_42 depending on Cell 1 band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 RSRP from Noc1 and Es1 / Noc1, the uncertainty in Cell 2 RSRP from Noc2 and Es2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.6.2 FDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / No2c: +3dB Ês3 / No2c: -1dB Reported relative RSRP values: (Cell 2 - Cell 1): ±6dB (Cell 3 - Cell 2): ±3dB |  |  | 0dB 0dB 0dB 0dB +0.8dB Via mapping Via mapping |  |  |  |  |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / No2c: +3dB Ês3 / No2c: -0.2dB (Cell 2 - Cell 1): RSRP_(x-1) to RSRP_(x+17) (Cell 3 - Cell 2): RSRP_(x-8) to RSRP_(x+1) |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 RSRP from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRP from Noc2 and Ês2 / Noc2, the uncertainty in Cell 3 RSRP from Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal and extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.6.2_1 FDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation (Rel 12 and forward) |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Es1 / Noc1: -4dB Es2 / No2c: +3dB Es3 / No2c: -1dB Reported relative RSRP values: (Cell 2 - Cell 1): ±4.5dB for normal conditions and ±6dB for extreme conditions (Cell 3 - Cell 2): ±3dB |  |  | 0dB 0dB 0dB 0dB +0.8dB Via mapping Via mapping |  |  |  |  |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Es1 / Noc1: -4dB Es2 / No2c: +3dB Es3 / No2c: -0.2dB Normal condition: (Cell 2 - Cell 1): RSRP_(x+2) to RSRP_(x+15) (Cell 3 - Cell 2): RSRP_(x-8) to RSRP_(x+1) Extreme condition: (Cell 2 - Cell 1): RSRP_(x-1) to RSRP_(x+17) (Cell 3 - Cell 2): RSRP_(x-8) to RSRP_(x+1) |  |
| 9.1.7.1 TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation |  |  | Same as 9.1.6.1 |  |  | Same as 9.1.6.1 |  |  |  |  |  |  | Same as 9.1.6.1 |  |
| 9.1.7.1_1 TDD Absolute RSRP Accuracy E-UTRA for Carrier Aggregation (Rel 12 and forward) |  |  | Same as 9.1.6.1_1 |  |  | Same as 9.1.6.1_1 |  |  |  |  |  |  | Same as 9.1.6.1_1 |  |
| 9.1.7.2 TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation |  |  | Same as 9.1.6.2 |  |  | Same as 9.1.6.2 |  |  |  |  |  |  | Same as 9.1.6.2 |  |
| 9.1.7.2_1 TDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation (Rel 12 and forward) |  |  | Same as 9.1.6.2_1 |  |  | Same as 9.1.6.2_1 |  |  |  |  |  |  | Same as 9.1.6.2_1 |  |
| 9.1.8.1 FDD Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with Non-MBSFN ABS |  |  | Test 1: Noc: -106.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB Reported RSRP values: ±6dB Test 2: Noc: -88.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported RSRP values: ±8dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported RSRP values: ±6dB |  |  | Test 1: 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB +0.95dB Via mapping Test 3: 0dB 0dB +0.95dB Via mapping |  |  |  |  |  |  | Test 1: Noc: -106.0Bm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB RSRP_25 to RSRP_40 Test 2: Noc: -88.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRP_40 to RSRP_59 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRP_14 to RSRP_29 RSRP_15 to RSRP_30 RSRP_16 to RSRP_31 RSRP_17 to RSRP_32 depending on operating band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. In all cases the RSRP values are 3dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.8.1_1 FDD Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) (Rel-12 and forward) |  |  | Test 1: Noc: -106.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB Reported RSRP values: ±6dB Test 2: Noc: -88.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported RSRP values: ±8dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported RSRP values: ±6dB |  |  | Test 1: 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB +0.95dB Via mapping Test 3: 0dB 0dB +0.95dB Via mapping |  |  |  |  |  |  | Test 1: Noc: -106.0Bm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB RSRP_26 to RSRP_39 Test 2: Noc: -88.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRP_40 to RSRP_59 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRP_15 to RSRP_27 RSRP_16 to RSRP_28 RSRP_17 to RSRP_29 RSRP_18 to RSRP_30 RSRP_19 to RSRP_31 depending on operating band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. The RSRP values are recalculated for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.8.2 FDD Relative RSRP Accuracy under Time Domain Measurement Resource Restriction with Non-MBSFN ABS |  |  | Test 1: Noc: -106.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB Reported relative RSRP values: ±2dB Test 2: Noc: -88.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported relative RSRP values: ±3dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported relative RSRP values: ±3dB |  |  | Test 1: 0dB 0dB +0.8dB Via mapping Test 2: 0dB 0dB +0.95dB Via mapping Test 3: 0dB 0dB +0.95dB Via mapping |  |  |  |  |  |  | Test 1: Noc: -106.0Bm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -1.2dB RSRP_x-10 to RSRP_x-4 Test 2: Noc: -88.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRP_x-13 to RSRP_x-4 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRP_x-13 to RSRP_x-4 |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc, Ês1 / Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. For Test 1 extreme conditions allow 1dB wider at each end. Test 2, Test 3 RSRP values for extreme conditions are the same for normal conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.9.1 TDD Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with Non-MBSFN ABS |  |  | Same as 9.1.8.1 |  |  | Same as 9.1.8.1 |  |  |  |  |  |  | Same as 9.1.8.1 |  |
| 9.1.9.1_1 TDD Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) (Rel 12 and forward) |  |  | Test 1: Noc: -106.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB Reported RSRP values: ±6dB Test 2: Noc: -88.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported RSRP values: ±8dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported RSRP values: ±6dB |  |  | Test 1: 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB +0.95dB Via mapping Test 3: 0dB 0dB +0.95dB Via mapping |  |  |  |  |  |  | Test 1: Noc: -106.0Bm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB RSRP_26 to RSRP_39 Test 2: Noc: -88.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRP_40 to RSRP_59 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRP_15 to RSRP_27 RSRP_16 to RSRP_28 RSRP_17 to RSRP_29 depending on operating band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. The RSRP values are recalculated for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.9.2 TDD Relative RSRP Accuracy under Time Domain Measurement Resource Restriction with Non-MBSFN ABS |  |  | Same as 9.1.8.2 |  |  | Same as 9.1.8.2 |  |  |  |  |  |  | Same as 9.1.8.2 |  |
| 9.1.10.1 FDD Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with MBSFN ABS |  |  | Test 1: Noc: -106.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB Reported RSRP values: ±6dB Test 2: Noc: -88.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported RSRP values: ±8dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported RSRP values: ±6dB |  |  | Test 1: 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB +0.95dB Via mapping Test 3: 0dB 0dB +0.95dB Via mapping |  |  |  |  |  |  | Test 1: Noc: -106.0Bm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB RSRP_25 to RSRP_40 Test 2: Noc: -88.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRP_40 to RSRP_59 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRP_14 to RSRP_29 RSRP_15 to RSRP_30 RSRP_16 to RSRP_31 RSRP_17 to RSRP_32 depending on operating band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. In all cases the RSRP values are 3dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.10.1_1 FDD Absolute RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Rel-12 and forward) |  |  | Test 1: Noc: -106.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB Reported RSRP values: ±6dB Test 2: Noc: -88.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported RSRP values: ±8dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported RSRP values: ±6dB |  |  | Test 1: 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB +0.95dB Via mapping Test 3: 0dB 0dB +0.95dB Via mapping |  |  |  |  |  |  | Test 1: Noc: -106.0Bm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB RSRP_26 to RSRP_40 Test 2: Noc: -88.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRP_39 to RSRP_59 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRP_15 to RSRP_27 RSRP_16 to RSRP_28 RSRP_17 to RSRP_29 RSRP_18 to RSRP_30 RSRP_19 to RSRP_31 depending on operating band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. The RSRP values are recalculated for extreme conditions |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.10.2 FDD Relative RSRP Accuracy under Time Domain Measurement Resource Restriction with MBSFN ABS |  |  | Test 1: Noc: -106.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB Reported relative RSRP values: ±2dB Test 2: Noc: -88.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported relative RSRP values: ±3dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported relative RSRP values: ±3dB |  |  |  |  | Test 1: 0dB 0dB +0.8dB Via mapping Test 2: 0dB 0dB +0.95dB Via mapping Test 3: 0dB 0dB +0.95dB Via mapping |  |  |  |  | Test 1: Noc: -106.0Bm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -1.2dB RSRP_x-10 to RSRP_x-4 Test 2: Noc: -88.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRP_x-13 to RSRP_x-4 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRP_x-13 to RSRP_x-4 |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc, Ês1 / Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. For Test 1 extreme conditions allow 1dB wider at each end. Test 2, Test 3 RSRP values for extreme conditions are the same for normal conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.11.1 TDD Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with MBSFN ABS |  |  | Same as 9.1.10.1 |  |  |  |  | Same as 9.1.10.1 |  |  |  |  | Same as 9.1.10.1 |  |
| 9.1.11.1_1 TDD Absolute RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Rel 12 and forward) |  |  | Test 1: Noc: -106.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB Reported RSRP values: ±6dB Test 2: Noc: -88.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported RSRP values: ±8dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported RSRP values: ±6dB |  |  |  |  | Test 1: 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB +0.95dB Via mapping Test 3: 0dB 0dB +0.95dB Via mapping |  |  |  |  | Test 1: Noc: -106.0Bm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB RSRP_26 to RSRP_40 Test 2: Noc: -88.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRP_39 to RSRP_59 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRP_15 to RSRP_27 RSRP_16 to RSRP_28 RSRP_17 to RSRP_29 depending on operating band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. The RSRP values are recalculated for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.11.2 TDD Relative RSRP Accuracy under Time Domain Measurement Resource Restriction with MBSFN ABS |  |  | Same as 9.1.10.2 |  |  |  |  | Same as 9.1.10.2 |  |  |  |  | Same as 9.1.10.2 |  |
| 9.1.12.1 FDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz |  |  | Same as 9.1.6.1 |  |  |  |  | Same as 9.1.6.1 |  |  |  |  | Same as 9.1.6.1 |  |
| 9.1.12.1_1 Measurement Performance Requirements / FDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz (Rel 12 and forward) |  |  | Same as 9.1.6.1_1 |  |  |  |  | Same as 9.1.6.1_1 |  |  |  |  | Same as 9.1.6.1_1 |  |
| 9.1.12.2 FDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz |  |  | Same as 9.1.6.2 |  |  |  |  | Same as 9.1.6.2 |  |  |  |  | Same as 9.1.6.2 |  |
| 9.1.12.2_1 FDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz (Rel 12 and forward) |  |  | Same as 9.1.6.2_1 |  |  |  |  | Same as 9.1.6.2_1 |  |  |  |  | Same as 9.1.6.2_1 |  |
| 9.1.13.1 TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz |  |  | Same as 9.1.6.1 |  |  |  |  | Same as 9.1.6.1 |  |  |  |  | Same as 9.1.6.1 |  |
| 9.1.13.1_1 TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz (Rel 12 and forward) |  |  | Same as 9.1.6.1_1 |  |  |  |  | Same as 9.1.6.1_1 |  |  |  |  | Same as 9.1.6.1_1 |  |
| 9.1.13.2 TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz |  |  | Same as 9.1.6.2 |  |  |  |  | Same as 9.1.6.2 |  |  |  |  | Same as 9.1.6.2 |  |
| 9.1.13.2_1 TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz (Rel 12 and forward) |  |  | Same as 9.1.12.2_1 |  |  |  |  | Same as 9.1.12.2_1 |  |  |  |  | Same as 9.1.12.2_1 |  |
| 9.1.14.1 FDD Intra Frequency Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) |  |  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -1.5dB Reported RSRP values: ±6dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -4.0dB Reported RSRP values: ±8dB Test 3: Noc: -116 to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -4.0dB Reported RSRP values: ±6dB |  |  |  |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB 0dB +1.0dB Via mapping |  |  |  |  | Test 1: Noc: -106.0dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -1.5dB RSRP_25 to RSRP_41 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -3.0dB RSRP_40 to RSRP_59 Test 3: Noc: -116 to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -3.0dB RSRP_14 to RSRP_29 RSRP_15 to RSRP_30 RSRP_15 to RSRP_31 RSRP_16 to RSRP_31 RSRP_17 to RSRP_32 RSRP_17 to RSRP_33 depending on operating band |  |
| 9.1.14.1_1 FDD Intra Frequency Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) (Rel 12 and forward) |  |  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -1.5dB Reported RSRP values: normal: ±4.5dB Reported RSRP values extreme: ±9dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -4.0dB Reported RSRP values normal: ±8dB Reported RSRP values extreme: ±11dB Test 3: Noc: -116 to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -4.0dB Reported RSRP values normal: ±4.5dB Reported RSRP values extreme: ±9dB |  |  |  |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB 0dB +1.0dB Via mapping |  |  |  |  | Test 1: Noc: -106.0dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -1.5dB RSRP_27 to RSRP_39 RSRP_22 to RSRP_44 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -3.0dB RSRP_40 to RSRP_59 RSRP_37 to RSRP_62 Test 3: Noc: -116 to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -3.0dB RSRP_15 to RSRP_28 RSRP_16 to RSRP_28 RSRP_16 to RSRP_29 RSRP_17 to RSRP_29 RSRP_17 to RSRP_30 RSRP_18 to RSRP_31 RSRP_19 to RSRP_31 depending on operating band RSRP_11 to RSRP_32 RSRP_11 to RSRP_33 RSRP_12 to RSRP_33 RSRP_12 to RSRP_34 RSRP_13 to RSRP_34 RSRP_14 to RSRP_35 RSRP_14 to RSRP_36 depending on operating band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.14.2 FDD Intra Frequency Relative RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) |  |  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -1.5dB Reported relative RSRP values: ±2dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -4.0dB Reported relative RSRP values: ±3dB Test 3: Noc: -116 to -112.5 dBm/15kHz depending on frequency band Ês1 / Noc: +4.0dB Ês2 / Noc: -3.0dB Ês3 / Noc: -4.0dB Reported relative RSRP values: ±3dB |  |  |  |  | Test 1: 0dB 0dB 0dB +1dB Via mapping Test 2: 0dB 0dB 0dB +1dB Via mapping Test 3: 0dB 0dB 0dB +1dB Via mapping |  |  |  |  | Test 1: Noc: -106.0dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -0.5dB RSRP_x-8 to RSRP_x-2 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -3.0dB RSRP_x-12 to RSRP_x-3 Test 3: Noc: -116 to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +4.0dB Ês2 / Noc: -3.0dB Ês3 / Noc: -3.0dB RSRP_x-12 to RSRP_x-3 |  |
| 9.1.15.1 TDD Intra Frequency Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) |  |  | Same as 9.1.14.1 |  |  |  |  | Same as 9.1.14.1 |  |  |  |  | Same as 9.1.14.1 |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc and Ês2 / Noc, the uncertainty in Cell 3 RSRP from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 3dB wider at each end in all cases. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.15.1_1 TDD Intra Frequency Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) (Rel 12 and forward) |  |  | Same as 9.1.14.1_1 |  |  |  |  | Same as 9.1.14.1_1 |  |  |  |  | Same as 9.1.14.1_1 |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. For extreme conditions,in test case 1 and 3 the RSRP values are 4.5 dB wider at each end , in test case 2 the RSRP values are 3 dB wider at each end |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.15.2 TDD Intra Frequency Relative RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) |  |  | Same as 9.1.14.2 |  |  |  |  | Same as 9.1.14.2 |  |  |  |  | Same as 9.1.14.2 |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 3 RSRP from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 1dB wider at each end for Test 1, the same than for normal conditions for Test 2, Test 3. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.16.1 FDD Intra Frequency Absolute RSRP Accuracy for 5MHz Bandwidth |  |  | Test 1: Noc: -103dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported relative RSRP values: ±3dB Test 2: Noc: -83dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported relative RSRP values: ±3dB Test 3: Noc: -109.5dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported relative RSRP values: ±3dB |  |  |  |  | Test 1: -1.0dB 0 dB +1.0dB Via mapping Test 2: -1.0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +0.8dB Via mapping |  |  |  |  | Test 1: Noc: -104.0dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_31 to RSRP_46 Test 2: Noc: -84dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_49 to RSRP_68 Test 3: Noc: -109.5dBm /15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: -0.2dB RSRP_23 to RSRP_38 |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. In all cases the RSRP values are 3dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.16.1_1 FDD Intra Frequency Absolute RSRP Accuracy for 5MHz Bandwidth (Rel 12 and forward) |  |  | Test 1: Noc: -103dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -83dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -109.5dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported RSRP values: ±4.5dB |  |  |  |  | Test 1: -1.0dB 0 dB +1.0dB Via mapping Test 2: -1.0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +0.8dB Via mapping |  |  |  |  | Test 1: Noc: -104.0dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_32 to RSRP_45 Test 2: Noc: -84dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_49 to RSRP_68 Test 3: Noc: -109.5dBm /15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: -0.2dB RSRP_25 to RSRP_37 |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. The RSRP values are recalculated for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.16.2 FDD Intra Frequency Relative Accuracy of RSRP for 5MHz Bandwidth |  |  | Test 1: Noc: -103dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported relative RSRP values: ±3dB Test 2: Noc: -83dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported relative RSRP values: ±3dB Test 3: Noc: -109.5.5dBm /15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported relative RSRP values: ±3dB |  |  |  |  | Test 1: 0 dB 0 dB +1.0dB Via mapping Test 2: -1.0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +1.0dB Via mapping |  |  |  |  | Test 1: Noc: -103 dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-9 to RSRP_x+1 Test 2: Noc: -84dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-9 to RSRP_x+1 Test 3: Noc: -109.5dBm /15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: 0dB RSRP_x-8 to RSRP_x+2 |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc, Ês1 / Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for both normal and extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.17.1 FDD Inter Frequency Absolute RSRP Accuracy for 5MHz Bandwidth |  |  | Test 1: Noc1: -85.65dBm/15kHz Ês1 / Noc1: +10.00dB Noc2: -85.65dBm/15kHz Ês2 / Noc2: +10.00dB Reported RSRP values: ±8dB Test 2: Noc1: (Noc2 +8dB) Ês1 / Noc1: +13.00dB Noc2: -110.5dBm /15kHz Ês2 / Noc2: -4.00dB Reported RSRP values: ±6dB |  |  |  |  | Test 1: -0.6dB 0dB -0.6dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  | Test 1: Noc1: -86.25dBm/15kHz Ês1 / Noc1: +10.00dB Noc2: -86.25dBm/15kHz Ês2 / Noc2: +10.00dB RSRP_55 to RSRP_74 Test 2: Noc1: (Noc2 +8dB) Ês1 / Noc1: +13.00dB Noc2: -110.5dBm /15kHz Ês2 / Noc: -3.20dB RSRP_19 to RSRP_34 |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. In all cases the RSRP values are 3dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.17.2 FDD - FDD Inter Frequency Relative Accuracy of RSRP for 5MHz Bandwidth |  |  | Test 1: Noc1: -85.65dBm/15kHz Noc2: -85.65dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +10dB Reported relative RSRP values: ±6dB Test 2: Noc1: -102.5dBm/15kHz Noc2: -110.5dBm/15kHz Ês1 / Noc1: +13dB Ês2 / Noc2: -4.0dB Reported relative RSRP values: ±6dB |  |  |  |  | Test 1: -0.6dB -0.6dB 0dB 0dB Via mapping Test 2: -1.0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  | Test 1: Noc1: -86.25dBm/15kHz Noc2: -86.25dBm/15kHz Ês1 / Noc: +10dB Ês2 / Noc: +10dB RSRP_(x-9) to RSRP_(x+9) Test 2: Noc1: -103.5dBm/15kHz Noc2: -100.5dBm /15kHz Ês1 / Noc1: +13dB Ês2 / Noc2: -3.2dB RSRP_(x-32) to RSRP_(x-16) |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc1 and Ês1 / Noc1 and Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for both normal and extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.18.1 FDD Absolute RSRP Accuracy for E-UTRA for Carrier Aggregation for 10MHz + 5MHz |  |  | Same as 9.1.6.1 |  |  |  |  | Same as 9.1.6.1 |  |  |  |  | Same as 9.1.6.1 |  |
| 9.1.18.2 FDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation for 10MHz + 5MHz |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -1dB Reported relative RSRP values: (Cell 2 - Cell 1): ±6dB (Cell 3 - Cell 2): ±3dB |  |  |  |  | 0dB 0dB 0dB 0dB +0.8dB Via mapping Via mapping |  |  |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -0.2dB (Cell 2 - Cell 1): RSRP_(x-1) to RSRP_(x+17) (Cell 3 - Cell 2): RSRP_(x-8) to RSRP_(x+1) |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 RSRP from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRP from Noc2 and Ês2 / Noc2, the uncertainty in Cell 3 RSRP from Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal and extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.18.2_1 FDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation for 10MHz + 5MHz (Rel-12 and forward) |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -1dB Reported relative RSRP values: (Cell 2 - Cell 1): ±4.5dB normal condition ±6dB normal condition (Cell 3 - Cell 2): ±3dB normal and extreme condition |  |  |  |  | 0dB 0dB 0dB 0dB +0.8dB Via mapping Via mapping |  |  |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -0.2dB (Cell 2 - Cell 1): RSRP_(x+2) to RSRP_(x+15) RSRP_(x-1) to RSRP_(x+17) (Cell 3 - Cell 2): RSRP_(x-8) to RSRP_(x+1) |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 RSRP from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRP from Noc2 and Ês2 / Noc2, the uncertainty in Cell 3 RSRP from Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.19.1 TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 10MHz + 5MHz |  |  | Same as 9.1.6.1 |  |  |  |  | Same as 9.1.6.1 |  |  |  |  | Same as 9.1.6.1 |  |
| 9.1.19.2 TDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation for 10MHz + 5MHz |  |  | Same as 9.1.18.2 |  |  |  |  | Same as 9.1.18.2 |  |  |  |  | Same as 9.1.18.2 |  |
| 9.1.19.2_1 TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 10MHz + 5MHz (Rel-12 and forward) |  |  | Same as 9.1.18.2_1 |  |  |  |  | Same as 9.1.18.2_1 |  |  |  |  | Same as 9.1.18.2_1 |  |
| 9.1.20.1 FDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 5MHz + 5MHz bandwidth |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -1dB Reported RSRP values: ±6dB for normal conditions and ±9dB for extreme conditions |  |  |  |  | 0dB 0dB 0dB 0dB 0dB Via mapping Via mapping |  |  |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -1dB Cell 1: RSRP_12 to RSRP_27 RSRP_13 to RSRP_28 RSRP_13 to RSRP_29 RSRP_14 to RSRP_29 RSRP_15 to RSRP_30 RSRP_15 to RSRP_31 depending on operating band Cell 2: RSRP_20 to RSRP_35 RSRP_21 to RSRP_36 RSRP_21 to RSRP_37 RSRP_22 to RSRP_37 RSRP_23 to RSRP_38 RSRP_23 to RSRP_39 depending on Cell 1 band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 RSRP from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRP from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. In all cases the RSRP values are 3dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.20.2 FDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation for 5MHz + 5MHz |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -1dB Reported relative RSRP values: (Cell 2 - Cell 1): ±6dB (Cell 3 - Cell 2): ±3dB |  |  |  |  | 0dB 0dB 0dB 0dB +0.8dB Via mapping Via mapping |  |  |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -0.2dB (Cell 2 - Cell 1): RSRP_(x-1) to RSRP_(x+17) (Cell 3 - Cell 2): RSRP_(x-8) to RSRP_(x+1) |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 RSRP from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRP from Noc2 and Ês2 / Noc2, the uncertainty in Cell 3 RSRP from Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal and extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.21.1 TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 5MHz + 5MHz bandwidth |  |  | Same as 9.1.20.1 |  |  |  |  | Same as 9.1.20.1 |  |  |  |  | Same as 9.1.20.1 |  |
| 9.1.21.2 TDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation for 5MHz + 5MHz |  |  | Same as 9.1.20.2 |  |  |  |  | Same as 9.1.20.2 |  |  |  |  | Same as 9.1.20.2 |  |
| 9.1.22 FDD-TDD RSRP Accuracy E-UTRA for Carrier Aggregation with PCell in FDD |  |  | Same as 9.1.37 Cell 1 and Cell 2 absolute RSRP; and Cell 2 -Cell 1 relative RSRP and Cell 3- Cell 2 relative RSRP |  |  |  |  | Same as 9.1.37 Cell 1, Cell 2 and Cell 3 |  |  |  |  | Same as 9.1.37 Cell 1 and Cell 2 absolute RSRP; and Cell 2 -Cell 1 relative RSRP and Cell 3- Cell 2 relative RSRP |  |
| 9.1.23 FDD-TDD RSRP Accuracy E-UTRA for Carrier Aggregation with PCell in TDD |  |  | Same as 9.1.37 Cell 1 and Cell 2 absolute RSRP; and Cell 2 -Cell 1 relative RSRP and Cell 3- Cell 2 relative RSRP |  |  |  |  | Same as 9.1.37 Cell 1, Cell 2 and Cell 3 |  |  |  |  | Same as 9.1.37 Cell 1 and Cell 2 absolute RSRP; and Cell 2 -Cell 1 relative RSRP and Cell 3- Cell 2 relative RSRP |  |
| 9.1.24.1 TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz + 10MHz |  |  | Noc1: -117dBm or -116dBm or -115dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / No2c: +3dB Ês3 / No2c: -1dB Reported RSRP values: ±6dB for normal conditions and ±9dB for extreme conditions |  |  |  |  | 0dB 0dB 0dB 0dB 0dB Via mapping Via mapping |  |  |  |  | Noc1: -117dBm or -116dBm or -115dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / No2c: +3dB Ês3 / No2c: -1dB Cell 1: RSRP_12 to RSRP_27 RSRP_13 to RSRP_28 RSRP_14 to RSRP_29 depending on operating band Cell 2: RSRP_20 to RSRP_35 RSRP_21 to RSRP_36 RSRP_22 to RSRP_37 depending on Cell 1 band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 RSRP from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRP from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. In all cases the RSRP values are 3dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.24.2_1 TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz + 10MHz (Rel-12 and forward) |  |  | Noc1: -117dBm or -116dBm or -115dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -1dB Reported relative RSRP values: (Cell 2 - Cell 1): ±4.5dB (Cell 3 - Cell 2): ±3dB |  |  |  |  | 0dB 0dB 0dB 0dB +0.8dB Via mapping Via mapping |  |  |  |  | Noc1: -117dBm or -116dBm or -115dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -0.2dB (Cell 2 - Cell 1): RSRP_(x+2) to RSRP_(x+15) (Cell 3 - Cell 2): RSRP_(x-8) to RSRP_(x+1) |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 RSRP from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRP from Noc2 and Ês2 / Noc2, the uncertainty in Cell 3 RSRP from Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. In all cases the RSRP values are 1.5 dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.24.1_1 TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz + 10MHz (Rel-12 and forward) |  |  | Noc1: -117dBm or -116dBm or -115dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / No2c: +3dB Ês3 / No2c: -1dB Reported RSRP values: ±4.5dB for normal conditions and ±9dB for extreme conditions | 0dB 0dB 0dB 0dB 0dB Via mapping Via mapping |  |  |  |  |  | Noc1: -117dBm or -116dBm or -115dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / No2c: +3dB Ês3 / No2c: -1dB Cell 1: RSRP_13 to RSRP_26 RSRP_14 to RSRP_27 RSRP_15 to RSRP_28 depending on operating band Cell 2: RSRP_21 to RSRP_34 RSRP_22 to RSRP_35 RSRP_23 to RSRP_36 depending on Cell 1 band |  |  |  |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 RSRP from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRP from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. By using separate calculation for normal and extreme condition, all cases the RSRP values are 4dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.24.2 TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz + 10MHz |  |  | Noc1: -117dBm or -116dBm or -115dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -1dB Reported relative RSRP values: (Cell 2 - Cell 1): ±6dB (Cell 3 - Cell 2): ±3dB | 0dB 0dB 0dB 0dB +0.8dB Via mapping Via mapping |  |  |  |  |  | Noc1: -117dBm or -116dBm or -115dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -0.2dB (Cell 2 - Cell 1): RSRP_(x-1) to RSRP_(x+17) (Cell 3 - Cell 2): RSRP_(x-8) to RSRP_(x+1) |  |  |  |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 RSRP from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRP from Noc2 and Ês2 / Noc2, the uncertainty in Cell 3 RSRP from Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal and extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.25 FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal |  |  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported absolute RSRP values: Normal condtion: ±4.5dB Extreme condtion: ±9dB Reported relative RSRP values: ±3dB | Test 1: -1.0dB 0dB +1.0dB Via mapping |  |  |  |  |  | Test 1: Noc: -107.0dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_29 to RSRP_42 RSRP_25 to RSRP_46 RSRP_Cell 1 - 9 to RSRP_Cell 1 + 1 |  |  |  |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell1 and Cell 2 RSRP from Noc, Ês1 / Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.26 TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal |  |  | Same as 9.1.25 | Same as 9.1.25 |  |  |  |  |  | Same as 9.1.25 |  |  |  |  |
|  |  |  | Same as 9.1.25 |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.27 FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal |  |  | Test 1: Noc1: = Noc2 +6dB Noc2: -115dBm to -111.5dBm /15kHz depending on operating band Ês1 / Noc1: 13dB Ês2 / Noc2: -6dB Reported absolute RSRP values: Normal condtion: ±4.5dB Extreme condtion: ±9dB Reported relative RSRP values: Normal condtion: ±4.5dB Extreme condtion: ±6dB | Test 1: 0dB 0dB 0dB +0.8dB Via mapping Via mapping Via mapping |  |  |  |  |  | Test 1: Noc1: = Noc2 +6dB Noc2: -115dBm to -111.5dBm /15kHz depending on operating band Ês1 / Noc1: 13dB Ês2 / Noc2: -5.2dB Cell 2, absolute: RSRP_14 to RSRP_26 RSRP_15 to RSRP_27 RSRP_15 to RSRP_27 RSRP_16 to RSRP_28 RSRP_16 to RSRP_28 RSRP_17 to RSRP_29 RSRP_18 to RSRP_30 depending on operating band Cell 2, absolute: RSRP_10 to RSRP_31 RSRP_10 to RSRP_31 RSRP_11 to RSRP_32 RSRP_11 to RSRP_32 RSRP_12 to RSRP_33 RSRP_13 to RSRP_34 RSRP_13 to RSRP_34 depending on operating band Relative: RSRP_(x-31) to RSRP_(x-18) RSRP_(x-33) to RSRP_(x-17) |  |  |  |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty from all applicable Noc and Ês / Noc values, the allowed UE reporting accuracy, and the UE mapping function. Where extreme condition UE accuracies differ from normal condition accuracies, reported values are recalculated. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.28 TDD—TDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal |  |  | Same as 9.1.27 | Same as 9.1.27 |  |  |  |  |  | Same as 9.1.27 |  |  |  |  |
|  | Same as 9.1.27 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.29 FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal |  |  | Test 1: Noc: -116dBm to -112.5dBm /15kHz depending on operating band CRS Ês1 / Noc: 3dB CSI-RS Ês1 / Noc: 9dB CRS Ês2 / Noc: -1dB CSI-RS Ês2 / Noc: 5dB Reported absolute CSI-RSRP values: Normal condtion: ±4.5dB Extreme condtion: ±9dB Reported relative CSI-RSRP values: Normal condition: ±2dB Extreme condition: ±3dB | Test 1: 0 dB 0 dB 0 dB 0 dB +0.8 dB Via mapping |  |  |  |  |  | Test 1: Noc: -116dBm to -112.5dBm /15kHz depending on operating band CRS Ês1 / Noc: 3dB CSI-RS Ês1 / Noc: 9dB CRS Ês2 / Noc: -1dB CSI-RS Ês2 / Noc: 5.8dB CSI-RSRP_24 to CSI-RSRP_43 CSI-RSRP_20 to CSI-RSRP_47 CSI-RSRP_Cell 1 - 7 to CSI-RSRP_Cell 1 - 1 CSI-RSRP_Cell 1 - 8 to CSI-RSRP_Cell 1 + 1 |  |  |  |  |
|  |  |  | The derivation of the CSI-RSRP values takes into account the uncertainty in Cell 1 and Cell 2 CSI-RSRP from Noc, CSI-RS Ês1 / Noc and CSI-RS Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.30 TDD intra-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal |  |  | Same as 9.1.29 | Same as 9.1.29 |  |  |  |  |  | Same as 9.1.29 |  |  |  |  |
| 9.1.31 FDD-FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal |  |  | Noc1: = Noc2 +6dB Noc2: -115dBm to -111.5dBm /15kHz depending on operating band CSI-RS Ês1 / Noc1: 13dB CSI-RS Ês2 / Noc2: 0dB Reported absolute CSI-RSRP values: Normal condtion: ±4.5dB Extreme condtion: ±9dB Reported relative CSI-RSRP values: Normal condtion: ±4.5dB Extreme condtion: ±6dB | 0dB 0dB 0dB +0.8dB Via mapping Via mapping Via mapping Via mapping |  |  |  |  |  | Noc1: = Noc2 +6dB Noc2: -115dBm to -111.5dBm /15kHz depending on operating band CSI-RS Ês1 / Noc1: 13dB CSI-RS Ês2 / Noc2: 0.8dB Cell 2: CSI_RSRP_20 to CSI_RSRP_32 CSI_RSRP_21 to CSI_RSRP_33 CSI_RSRP_21 to CSI_RSRP_33 CSI_RSRP_22 to CSI_RSRP_34 CSI_RSRP_22 to CSI_RSRP_34 CSI_RSRP_23 to CSI_RSRP_35 CSI_RSRP_24 to CSI_RSRP_36 CSI_RSRP_16 to CSI_RSRP_37 CSI_RSRP_16 to CSI_RSRP_37 CSI_RSRP_17 to CSI_RSRP_38 CSI_RSRP_17 to CSI_RSRP_38 CSI_RSRP_18 to CSI_RSRP_39 CSI_RSRP_19 to CSI_RSRP_40 CSI_RSRP_19 to CSI_RSRP_40 Cell 2 - Cell 1 CSI_RSRP_Cell 1 - 25 to CSI_RSRP_Cell 1 - 12 CSI_RSRP_Cell 1 - 27 to CSI_RSRP_Cell 1 - 11 |  |  |  |  |
|  |  |  | The derivation of the CSI-RSRP values takes into account the uncertainty in Cell 1 and Cell 2 CSI-RSRP from Noc, CSI-RS Ês1 / Noc and CSI-RS Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.33 FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal |  |  | Test 1: Noc1: -117dBm to -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: 3dB Ês3 / Noc2: -1dB Reported absolute RSRP values: Normal condtion: ±4.5dB Extreme condtion: ±9dB Reported relative RSRP values: (Cell 3 - Cell 2): ±3dB (Cell 3 - Cell 1): Normal condtion: ±4.5dB Extreme condtion: ±6dB | Test 1: 0dB 0dB 0dB 0dB +0.8dB Via mapping Via mapping Via mapping |  |  |  |  |  | Test 1: Noc1: -117dBm to -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: 3dB Ês3 / Noc2: -0.2dB Cell 3, absolute: RSRP_18 to RSRP_30 RSRP_19 to RSRP_31 RSRP_19 to RSRP_31 RSRP_20 to RSRP_32 RSRP_20 to RSRP_32 RSRP_21 to RSRP_33 RSRP_22 to RSRP_34 depending on operating band Cell 3, absolute: RSRP_14 to RSRP_35 RSRP_14 to RSRP_35 RSRP_15 to RSRP_36 RSRP_15 to RSRP_36 RSRP_16 to RSRP_37 RSRP_17 to RSRP_38 RSRP_17 to RSRP_38 depending on operating band Relative: RSRP_(x-8) to RSRP_(x+1) RSRP_(x-2) to RSRP_(x+12) RSRP_(x-4) to RSRP_(x+13) |  |  |  |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty from all applicable Noc and Ês / Noc values, the allowed UE reporting accuracy, and the UE mapping function. Where extreme condition UE accuracies differ from normal condition accuracies, reported values are recalculated. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.34 TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal |  |  | Same as 9.1.33 | Same as 9.1.33 |  |  |  |  | Same as 9.1.33 |  |  |  |  |  |
|  |  |  | Same as 9.1.33 |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.35 FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal |  |  | Noc1: -117dBm to -113.5dBm /15kHz depending on operating band Noc2: = Noc2 +1dB CSI-RS Ês1 / Noc1: 2dB CSI-RS Ês2 / Noc2: 9dB CSI-RS Ês3 / Noc3: 5dB Reported absolute CSI-RSRP values: Normal condtion: ±4.5dB Extreme condtion: ±9dB Reported intra-freq relative CSI-RSRP values: Normal condtion: ±2dB Extreme condtion: ±3dB Reported inter-freq relative CSI-RSRP values: Normal condtion: ±4.5dB Extreme condtion: ±6dB |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | The derivation of the CSI-RSRP values takes into account the uncertainty in Cell 1 and Cell 2 CSI-RSRP from Noc, CSI-RS Ês / Noc, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.36 TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal |  |  | Same as 9.1.35 |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.37 3DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation |  |  | Noc1: -117dBm or -116.5dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Noc3: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -1dB Ês4 / Noc3: +3dB Ês5 / Noc3: -1dB Reported absolute RSRP values: ±4.5dB for normal conditions and ±9dB for extreme conditions Reported relative RSRP values: (Cell 2 - Cell 1) and (Cell 4 - Cell 1): ±4.5dB for normal conditions ±6dB for extreme conditions (Cell 3 - Cell 2) and (Cell 5 - Cell 4): ±3dB |  |  | 0dB 0dB 0dB 0dB 0dB +0.8dB 0dB +0.8dB Via mapping Via mapping Via mapping |  |  |  |  |  | Noc1: -117dBm or -116.5dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Noc3: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -0.2dB Ês4 / Noc3: +3dB Ês5 / Noc3: -0.2dB Cell 1, absolute: RSRP_13 to RSRP_26 RSRP_14 to RSRP_26 RSRP_14 to RSRP_27 RSRP_15 to RSRP_27 RSRP_15 to RSRP_28 RSRP_16 to RSRP_29 RSRP_17 to RSRP_29 depending on operating band Cell 2 and Cell 4, absolute: RSRP_21 to RSRP_34 RSRP_22 to RSRP_34 RSRP_22 to RSRP_35 RSRP_23 to RSRP_35 RSRP_23 to RSRP_36 RSRP_24 to RSRP_37 RSRP_25 to RSRP_37 depending on Cell 1 band Relative: (Cell 2 - Cell 1) and (Cell 4 - Cell 1): RSRP_(x+2) to RSRP_(x+15) (Cell 3 - Cell 2) and (Cell 5 - Cell 4): RSRP_(x-8) to RSRP_(x+1) |  |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty from all applicable Noc and Ês / Noc values, the allowed UE reporting accuracy, and the UE mapping function. Where extreme condition UE accuracies differ from normal condition accuracies, reported values are recalculated. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.38 3DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation |  |  | Same as 9.1.37 |  |  | Same as 9.1.37 |  |  |  |  |  | Same as 9.1.37 |  |  |
| 9.1.39 3DL FDD RSRP for E-UTRAN in Carrier Aggregation |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Noc3: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -1dB Ês4 / Noc3: +3dB Ês5 / Noc3: -1dB Reported absolute RSRP values: ±6dB for normal conditions and ±9dB for extreme conditions Reported relative RSRP values: (Cell 2 - Cell 1) and (Cell 4 - Cell 1): ±6dB (Cell 3 - Cell 2) and (Cell 5 - Cell 4): ±3dB |  |  | 0dB 0dB 0dB 0dB 0dB +0.8dB 0dB +0.8dB Via mapping Via mapping Via mapping |  |  |  |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Noc3: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -0.2dB Ês4 / Noc3: +3dB Ês5 / Noc3: -0.2dB Cell 1, absolute: RSRP_12 to RSRP_27 RSRP_13 to RSRP_28 RSRP_13 to RSRP_29 RSRP_14 to RSRP_29 RSRP_15 to RSRP_30 RSRP_15 to RSRP_31 depending on operating band Cell 2 and Cell 4, absolute: RSRP_20 to RSRP_35 RSRP_21 to RSRP_36 RSRP_21 to RSRP_37 RSRP_22 to RSRP_37 RSRP_23 to RSRP_38 RSRP_23 to RSRP_39 depending on Cell 1 band Relative: (Cell 2 - Cell 1) and (Cell 4 - Cell 1): RSRP_(x-1) to RSRP_(x+17) (Cell 3 - Cell 2) and (Cell 5 - Cell 4): RSRP_(x-8) to RSRP_(x+1) |  |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty from all applicable Noc and Ês / Noc values, the allowed UE reporting accuracy, and the UE mapping function. Where extreme condition UE accuracies differ from normal condition accuracies, reported values are recalculated. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.39_1 3DL FDD RSRP for E-UTRAN in Carrier Aggregation (Rel-12 and forward) |  |  | Same as 9.1.37 |  |  | Same as 9.1.37 |  |  |  |  |  | Same as 9.1.37 |  |  |
| 9.1.40 3DL TDD RSRP for E-UTRAN in Carrier Aggregation |  |  | Same as 9.1.39 |  |  | Same as 9.1.39 |  |  |  |  |  | Same as 9.1.39 |  |  |
| 9.1.40_1 3DL TDD RSRP for E-UTRAN in Carrier Aggregation (Rel-12 and forward) |  |  | Same as 9.1.37 |  |  | Same as 9.1.37 |  |  |  |  |  | Same as 9.1.37 |  |  |
| 9.1.41.1 FD-FDD Intra Frequency Absolute RSRP Accuracy for UE category 0 |  |  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±7dB Test 2: Noc: -86dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±9dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported RSRP values: ±7dB |  |  | Test 1: -1.0dB 0dB +1.0dB Via mapping Test 2: -1.0dBdB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +0.8dB Via mapping |  |  |  |  |  | Test 1: Noc: -107.0dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_27 to RSRP_44 Test 2: Noc: -87dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_45 to RSRP_66 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +3.0dB Ês2 / Noc: -0.2dB RSRP_16 to RSRP_33 RSRP_17 to RSRP_34 RSRP_18 to RSRP_35 RSRP_19 to RSRP_36 depending on operating band |  |  |
| 9.1.41.2 FD-FDD Intra Frequency Relative RSRP Accuracy for UE category 0 |  |  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported relative RSRP values: ±4dB Test 2: Noc: -86dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported relative RSRP values: ±4dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported relative RSRP values: ±4dB |  |  | Test 1: 0 dB 0 dB +1.0dB Via mapping Test 2: -1.0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +1.0dB Via mapping |  |  |  |  |  | Test 1: Noc: -106 dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-10 to RSRP_x+2 Test 2: Noc: -87dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-10 to RSRP_x+2 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +3.0dB Ês2 / Noc: 0dB RSRP_x-9 to RSRP_x+3 |  |  |
| 9.1.42.1 HD-FDD Intra Frequency Absolute RSRP Accuracy for UE category 0 |  |  | Same as 9.1.41.1 |  |  | Same as 9.1.41.1 |  |  |  |  |  | Same as 9.1.41.1 |  |  |
| 9.1.42.2 HD-FDD Intra Frequency Relative RSRP Accuracy for UE category 0 |  |  | Same as 9.1.41.2 |  |  | Same as 9.1.41.2 |  |  |  |  |  | Same as 9.1.41.2 |  |  |
| 9.1.43.1 TDD Intra Frequency Absolute RSRP Accuracy for UE category 0 |  |  | Same as 9.1.41.1 |  |  | Same as 9.1.41.1 |  |  |  |  |  | Same as 9.1.41.1 |  |  |
| 9.1.43.2 TDD Intra Frequency Relative RSRP Accuracy for UE category 0 |  |  | Same as 9.1.41.2 |  |  | Same as 9.1.41.2 |  |  |  |  |  | Same as 9.1.41.2 |  |  |
| 9.1.44 4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation |  |  | Noc1: -117dBm or -116.5dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Noc3: = Noc1 +1dB Noc4: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -1dB Ês4 / Noc3: +3dB Ês5 / Noc3: -1dB Ês6 / Noc4: +3dB Ês7 / Noc4: -1dB Reported absolute RSRP values: ±4.5dB for normal conditions and ±9dB for extreme conditions Reported relative RSRP values: (Cell 2 - Cell 1) and (Cell 4 - Cell 1), (Cell 6 - Cell 1): ±4.5dB for normal conditions ±6dB for extreme conditions (Cell 3 - Cell 2) and (Cell 5 - Cell 4), (Cell 7 - Cell 6): ±3dB |  |  | 0dB 0dB 0dB 0dB 0dB 0dB +0.8dB 0dB +0.8dB 0dB +0.8dB Via mapping Via mapping Via mapping |  |  |  |  |  | Noc1: -117dBm or -116.5dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Noc3: = Noc1 +1dB Noc4: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -0.2dB Ês4 / Noc3: +3dB Ês5 / Noc3: -0.2dB Ês6 / Noc4: +3dB Ês7 / Noc4: -0.2dB Cell 1, absolute: RSRP_13 to RSRP_26 RSRP_14 to RSRP_26 RSRP_14 to RSRP_27 RSRP_15 to RSRP_27 RSRP_15 to RSRP_28 RSRP_16 to RSRP_29 RSRP_17 to RSRP_29 RSRP_20 to RSRP_32 depending on operating band Cell 2 and Cell 4, Cell 6, absolute: RSRP_21 to RSRP_34 RSRP_22 to RSRP_34 RSRP_22 to RSRP_35 RSRP_23 to RSRP_35 RSRP_23 to RSRP_36 RSRP_24 to RSRP_37 RSRP_25 to RSRP_37 RSRP_28 to RSRP_40 depending on Cell 1 band Relative: (Cell 2 - Cell 1) and (Cell 4 - Cell 1),(Cell 6 -Cell 1 ): RSRP_(x+2) to RSRP_(x+15) RSRP_(x-1) to RSRP_(x+17) (Cell 3 - Cell 2) and (Cell 5 - Cell 4), (Cell 7 - Cell 6): RSRP_(x-8) to RSRP_(x+1) The Test Tolerance values are taken from the analysis for the related 3DL Test case 9.1.37, and no separate analysis is needed. |  |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty from all applicable Noc and Ês / Noc values, the allowed UE reporting accuracy, and the UE mapping function. Where extreme condition UE accuracies differ from normal condition accuracies, reported values are recalculated. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.45 4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation |  |  | Same as 9.1.44 |  |  | Same as 9.1.44 |  |  |  |  |  | Same as 9.1.44 |  |  |
| 9.1.46 4DL FDD RSRP for E-UTRAN in Carrier Aggregation |  |  | Same as 9.1.44 |  |  | Same as 9.1.44 |  |  |  |  |  | Same as 9.1.44 |  |  |
| 9.1.47 4DL TDD RSRP for E-UTRAN in Carrier Aggregation |  |  | Same as 9.1.44 |  |  | Same as 9.1.44 |  |  |  |  |  | Same as 9.1.44 |  |  |
| 9.1.48 5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation |  |  | Noc1: -117dBm or -116.5dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Noc3: = Noc1 +1dB Noc4: = Noc1 +1dB Noc5: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -1dB Ês4 / Noc3: +3dB Ês5 / Noc3: -1dB Ês6 / Noc4: +3dB Ês7 / Noc4: -1dB Ês8 / Noc5: +3dB Ês9 / Noc5: -1dB Reported absolute RSRP values: ±4.5dB for normal conditions and ±9dB for extreme conditions Reported relative RSRP values: (Cell 2 - Cell 1) and (Cell 4 - Cell 1), (Cell 6 - Cell 1), (Cell 8 - Cell 1): ±4.5dB for normal conditions ±6dB for extreme conditions (Cell 3 - Cell 2) and (Cell 5 - Cell 4), (Cell 7 - Cell 6), (Cell 9 - Cell 8): ±3dB |  |  | 0dB 0dB 0dB 0dB 0dB 0dB 0dB +0.8dB 0dB +0.8dB 0dB +0.8dB 0dB +0.8dB Via mapping Via mapping Via mapping |  |  |  |  |  | Noc1: -117dBm or -116.5dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Noc3: = Noc1 +1dB Noc4: = Noc1 +1dB Noc5: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -0.2dB Ês4 / Noc3: +3dB Ês5 / Noc3: -0.2dB Ês6 / Noc4: +3dB Ês7 / Noc4: -0.2dB Ês8 / Noc5: +3dB Ês9 / Noc5: -0.2dB Cell 1, absolute: RSRP_13 to RSRP_26 RSRP_14 to RSRP_26 RSRP_14 to RSRP_27 RSRP_15 to RSRP_27 RSRP_15 to RSRP_28 RSRP_16 to RSRP_29 RSRP_17 to RSRP_29 RSRP_20 to RSRP_32 depending on operating band Cell 2 and Cell 4, Cell 6, absolute: RSRP_21 to RSRP_34 RSRP_22 to RSRP_34 RSRP_22 to RSRP_35 RSRP_23 to RSRP_35 RSRP_23 to RSRP_36 RSRP_24 to RSRP_37 RSRP_25 to RSRP_37 RSRP_28 to RSRP_40 depending on Cell 1 band Relative: (Cell 2 - Cell 1) and (Cell 4 - Cell 1),(Cell 6 -Cell 1), (Cell 8 - Cell 1): RSRP_(x+2) to RSRP_(x+15) RSRP_(x-1) to RSRP_(x+17) (Cell 3 - Cell 2) and (Cell 5 - Cell 4), (Cell 7 - Cell 6), (Cell 9 - Cell 8): RSRP_(x-8) to RSRP_(x+1) The Test Tolerance values are taken from the analysis for the related 3DL Test case 9.1.37, and no separate analysis is needed. |  |  |
| 9.1.49 5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation |  |  | Same as 9.1.48 |  |  | Same as 9.1.48 |  |  |  |  |  | Same as 9.1.48 |  |  |
| 9.1.50 5 DL FDD RSRP for E-UTRAN in Carrier Aggregation |  |  | Same as 9.1.48 |  |  | Same as 9.1.48 |  |  |  |  |  | Same as 9.1.48 |  |  |
| 9.1.51 5 DL TDD RSRP for E-UTRAN in Carrier Aggregation |  |  | Same as 9.1.48 |  |  | Same as 9.1.48 |  |  |  |  |  | Same as 9.1.48 |  |  |
| 9.1.52 FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA |  |  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported absolute RSRP values: Normal conditions: ±7dB Extreme conditions: ±10dB Reported relative RSRP values: Normal and extreme conditions: ±4dB Test 2: Noc: -86dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported absolute RSRP values: Normal conditions: ±9dB Extreme conditions: ±12dB Reported relative RSRP values: Normal and extreme conditions: ±4dB Test 3: Noc: -116dBm to -112.5 dBm/15kHz depending on operating band Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported absolute RSRP values: Normal conditions: ±7dB Extreme conditions: ±10dB Reported relative RSRP values: Normal and extreme conditions: ±4dB |  |  | Test 1: -1.0dB 0dB +1.0dB Via mapping Test 2: -1.0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +0.8dB Via mapping |  |  |  |  |  | Test 1: Noc: -107dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_27 to RSRP_44 RSRP_24 to RSRP_47 RSRP_Cell 1 - 10 to RSRP_Cell 1 + 2 Test 2: Noc: -87dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_45 to RSRP_66 RSRP_52 to RSRP_69 RSRP_Cell 1 - 10 to RSRP_Cell 1 + 2 Test 3: Noc: -116dBm to -112.5 dBm/15kHz depending on operating band Ês1 / Noc: +3.0dB Ês2 / Noc: -0.2dB RSRP_16 to RSRP_33 RSRP_16 to RSRP_33 RSRP_17 to RSRP_34 RSRP_17 to RSRP_34 RSRP_18 to RSRP_35 RSRP_19 to RSRP_36 RSRP_19 to RSRP_36 RSRP_13 to RSRP_36 RSRP_13 to RSRP_36 RSRP_14 to RSRP_37 RSRP_14 to RSRP_37 RSRP_15 to RSRP_38 RSRP_16 to RSRP_39 RSRP_16 to RSRP_39 RSRP_Cell 1 - 9 to RSRP_Cell 1 + 2 |  |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc, Ês1 / Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The absolute RSRP values given above are different for normal and extreme conditions. The relative RSRP values given above are for both normal and extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.53 HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA |  |  | Same as 9.1.52 |  |  | Same as 9.1.52 |  |  |  |  |  | Same as 9.1.52 |  |  |
| 9.1.54 TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA |  |  | Same as 9.1.52 |  |  | Same as 9.1.52 |  |  |  |  |  | Same as 9.1.52 |  |  |
| 9.1.55 FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell9.1.55 FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell |  |  | Test 1: Noc1: -117dBm to -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: 3dB Ês3 / Noc2: -1dB Reported absolute RSRP values: Normal condtion: ±4.5dB Extreme condtion: ±9dB Reported relative RSRP values: (Cell 3 - Cell 2): ±3dB (Cell 3 - Cell 1): Normal condtion: ±4.5dB Extreme condtion: ±6dB |  |  | Test 1: 0dB 0dB 0dB 0dB +0.8dB Via mapping Via mapping Via mapping |  |  |  |  |  | Test 1: Noc1: -117dBm to -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: 3dB Ês3 / Noc2: -0.2dB Cell 3, absolute: RSRP_18 to RSRP_31 RSRP_18 to RSRP_31 RSRP_19 to RSRP_32 RSRP_19 to RSRP_32 RSRP_20 to RSRP_33 RSRP_21 to RSRP_34 RSRP_22 to RSRP_34 depending on operating band Cell 3, absolute: RSRP_13 to RSRP_35 RSRP_14 to RSRP_36 RSRP_14 to RSRP_36 RSRP_15 to RSRP_37 RSRP_15 to RSRP_37 RSRP_16 to RSRP_38 RSRP_17 to RSRP_39 depending on operating band Relative: RSRP_(x-8) to RSRP_(x+1) RSRP_(x-3) to RSRP_(x+12) RSRP_(x-4) to RSRP_(x+14) |  |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty from all applicable Noc and Ês / Noc values, the allowed UE reporting accuracy, and the UE mapping function. Where extreme condition UE accuracies differ from normal condition accuracies, reported values are recalculated. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.56 FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell |  |  | Same as 9.1.55 |  |  | Same as 9.1.55 |  |  |  |  |  | Same as 9.1.55 |  |  |
| 9.1.57 FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB |  |  | Test 1: Noc: = -99dBm/15kHz Ês1 / Noc: -12dB Ês2 / Noc: -14dB Reported absolute RSRP values: Normal condtion: ±8dB Extreme condtion: ±11dB Reported relative RSRP values: Normal condtion: ±5dB Extreme condtion: ±5dB Test 2: Noc: = -79dBm/15kHz Ês1 / Noc: -12dB Ês2 / Noc: -14dB Reported absolute RSRP values: Normal condtion: ±10dB Extreme condtion: ±13dB Reported relative RSRP values: Normal condtion: ±5dB Extreme condtion: ±5dB Test 3: Noc: = -107dBm~-103.5dBm/15kHz depending on operating band Ês1 / Noc: -12dB Ês2 / Noc: -14dB Reported absolute RSRP values: Normal condtion: ±8dB Extreme condtion: ±11dB Reported relative RSRP values: Normal condtion: ±5dB Extreme condtion: ±5dB |  |  | Test 1: -0.3dB 0dB +0.1dB Via mapping Via mapping Test 2: -0.3dB 0dB +0.1dB Via mapping Via mapping Test 3: 0dB 0dB +0.1dB Via mapping Via mapping Via mapping |  |  |  |  |  | Test 1: Noc: = -99.3dBm/15kHz Ês1 / Noc: -12dB Ês2 / Noc: -13.9dB Cell 2, absolute: RSRP_18 to RSRP_37 RSRP_15 to RSRP_40 Relative: RSRP_(x-9) to RSRP_(x+5) RSRP_(x-9) to RSRP_(x+5) Test 2: Noc: = -79.3dBm/15kHz Ês1 / Noc: -12dB Ês2 / Noc: -13.9dB Cell 2, absolute: RSRP_36 to RSRP_59 RSRP_33 to RSRP_62 Relative: RSRP_(x-9) to RSRP_(x+5) RSRP_(x-9) to RSRP_(x+5) Test 3: Noc: = -107dBm~-103.5dBm/15kHz depending on operating band Ês1 / Noc: -12dB Ês2 / Noc: -13,9dB Cell 2, absolute: RSRP_10 to RSRP_29 RSRP_11 to RSRP_30 RSRP_11 to RSRP_30 RSRP_12 to RSRP_31 RSRP_12 to RSRP_31 RSRP_13 to RSRP_32 RSRP_14 to RSRP_33 depending on operating band Cell 2, absolute: RSRP_7 to RSRP_32 RSRP_8 to RSRP_33 RSRP_8 to RSRP_33 RSRP_9 to RSRP_34 RSRP_9 to RSRP_34 RSRP_10 to RSRP_35 RSRP_11 to RSRP_36 depending on operating band Relative: RSRP_(x-9) to RSRP_(x+5) RSRP_(x-9) to RSRP_(x+5) |  |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty from all applicable Noc and Ês / Noc values, the allowed UE reporting accuracy, and the UE mapping function. Where extreme condition UE accuracies differ from normal condition accuracies, reported values are recalculated. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.58 HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB |  |  | Same as 9.1.57 |  |  | Same as 9.1.57 |  |  |  |  |  | Same as 9.1.57 |  |  |
|  |  |  | Same as 9.1.57 |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.59 TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB |  |  | Same as 9.1.57 |  |  | Same as 9.1.57 |  |  |  |  |  | Same as 9.1.57 |  |  |
|  |  |  | Same as 9.1.57 |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.60 FS3 absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal with FDD PCell |  |  | Noc1: -117dBm or -116.5dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB CRS Ês1 / Noc1: -4dB CRS Ês2 / Noc2: +3dB CRS Ês3 / Noc2: -1dB CSI-RS Ês2 / Noc2: +9dB CSI-RS Ês3 / Noc2: +5dB Reported absolute RSRP values: ±4.5dB for normal conditions and ±9dB for extreme conditions Reported relative RSRP values: (Cell 3 - Cell 2): ±2dB for normal conditions and ±3dB for extreme conditions |  |  | 0dB 0dB 0dB 0dB 0dB 0dB +0.8dB Via mapping Via mapping |  |  |  |  |  | Noc1: -117dBm or -116.5dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB CRS Ês1 / Noc1: -4dB CRS Ês2 / Noc2: +3dB CRS Ês3 / Noc2: -1dB CSI-RS Ês2 / Noc2: +9dB CSI-RS Ês3 / Noc2: +5.8dB Cell 2, absolute: RSRP_27 to RSRP_40 RSRP_28 to RSRP_40 RSRP_28 to RSRP_41 RSRP_29 to RSRP_41 RSRP_29 to RSRP_42 RSRP_30 to RSRP_43 RSRP_31 to RSRP_43 depending on Cell 1 operating band Cell 3, absolute: RSRP_24 to RSRP_36 RSRP_25 to RSRP_37 RSRP_25 to RSRP_37 RSRP_26 to RSRP_38 RSRP_26 to RSRP_38 RSRP_27 to RSRP_39 RSRP_28 to RSRP_40 depending on Cell 1 operating band Relative: RSRP_(x-7) to RSRP_(x-1) |  |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty from all applicable Noc ,CRS Ês / Noc and CSI-RS Ês / Noc values, the allowed UE reporting accuracy, and the UE mapping function. Where extreme condition UE accuracies differ from normal condition accuracies, reported values are recalculated. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.61 FS3 absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal with TDD PCell |  |  | Same as 9.1.60 |  |  | Same as 9.1.60 |  |  |  |  |  | Same as 9.1.60 |  |  |
| 9.1.68 3DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes |  |  | Same as 9.1.37 |  |  | Same as 9.1.37 |  |  |  |  |  | Same as 9.1.37 |  |  |
| 9.1.69 4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes |  |  | Same as 9.1.44 |  |  | Same as 9.1.44 |  |  |  |  |  | Same as 9.1.44 |  |  |
| 9.1.70 5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes |  |  | Same as 9.1.48 |  |  | Same as 9.1.48 |  |  |  |  |  | Same as 9.1.48 |  |  |
| 9.1.71 6 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes |  |  | Noc1: -117dBm or -116.5dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Noc3: = Noc1 +1dB Noc4: = Noc1 +1dB Noc5: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -1dB Ês4 / Noc3: +3dB Ês5 / Noc3: -1dB Ês6 / Noc4: +3dB Ês7 / Noc4: -1dB Ês8 / Noc5: +3dB Ês9 / Noc5: -1dB Ês10 / Noc6: +3dB Ês11 / Noc6: -1dB Reported absolute RSRP values: ±4.5dB for normal conditions and ±9dB for extreme conditions Reported relative RSRP values: (Cell 2 – Cell 1) and (Cell 4 – Cell 1), (Cell 6 – Cell 1), (Cell 8 – Cell 1), (Cell 10 – Cell 1): ±4.5dB for normal conditions ±6dB for extreme conditions (Cell 3 - Cell 2) and (Cell 5 - Cell 4), (Cell 7 - Cell 6), (Cell 9 – Cell 8), (Cell 11 – Cell 10): ±3dB |  |  | 0dB 0dB 0dB 0dB 0dB 0dB 0dB +0.8dB 0dB +0.8dB 0dB +0.8dB 0dB +0.8dB 0dB +0.8dB Via mapping Via mapping Via mapping |  |  |  |  |  | Noc1: -117dBm or -116.5dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Noc3: = Noc1 +1dB Noc4: = Noc1 +1dB Noc5: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -0.2dB Ês4 / Noc3: +3dB Ês5 / Noc3: -0.2dB Ês6 / Noc4: +3dB Ês7 / Noc4: -0.2dB Ês8 / Noc5: +3dB Ês9 / Noc5: -0.2dB Ês10 / Noc6: +3dB Ês11 / Noc6: -0.2dB Cell 1, absolute: RSRP_13 to RSRP_26 RSRP_14 to RSRP_26 RSRP_14 to RSRP_27 RSRP_15 to RSRP_27 RSRP_15 to RSRP_28 RSRP_16 to RSRP_29 RSRP_17 to RSRP_29 depending on operating band Cell 2 and Cell 4, Cell 6, Cell 8, Cell 10 absolute: RSRP_21 to RSRP_34 RSRP_22 to RSRP_34 RSRP_22 to RSRP_35 RSRP_23 to RSRP_35 RSRP_23 to RSRP_36 RSRP_24 to RSRP_37 RSRP_25 to RSRP_37 depending on Cell 1 band Relative: (Cell 2 – Cell 1) and (Cell 4 – Cell 1),(Cell 6 –Cell 1), (Cell 8 – Cell 1), (Cell 10 – Cell 1): RSRP_(x+2) to RSRP_(x+15) RSRP_(x-1) to RSRP_(x+17) (Cell 3 - Cell 2) and (Cell 5 - Cell 4), (Cell 7 - Cell 6), (Cell 9 – Cell 8), (Cell 11 – Cell 10): RSRP_(x-8) to RSRP_(x+1) The Test Tolerance values are taken from the analysis for the related 3DL Test case 9.1.37, and no separate analysis is needed. |  |  |
| 9.1.72 7 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes |  |  | Noc1: -117dBm or -116.5dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Noc3: = Noc1 +1dB Noc4: = Noc1 +1dB Noc5: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -1dB Ês4 / Noc3: +3dB Ês5 / Noc3: -1dB Ês6 / Noc4: +3dB Ês7 / Noc4: -1dB Ês8 / Noc5: +3dB Ês9 / Noc5: -1dB Ês10 / Noc6: +3dB Ês11 / Noc6: -1dB Ês12 / Noc7: +3dB Ês13 / Noc7: -1dB Reported absolute RSRP values: ±4.5dB for normal conditions and ±9dB for extreme conditions Reported relative RSRP values: (Cell 2 – Cell 1) and (Cell 4 – Cell 1), (Cell 6 – Cell 1), (Cell 8 – Cell 1), (Cell 10 – Cell 1), (Cell 12 – Cell 1): ±4.5dB for normal conditions ±6dB for extreme conditions (Cell 3 - Cell 2) and (Cell 5 - Cell 4), (Cell 7 - Cell 6), (Cell 9 – Cell 8), (Cell 11 – Cell 10), (Cell 13 – Cell 12): ±3dB |  |  | 0dB 0dB 0dB 0dB 0dB 0dB 0dB +0.8dB 0dB +0.8dB 0dB +0.8dB 0dB +0.8dB 0dB +0.8dB 0dB +0.8dB Via mapping Via mapping Via mapping |  |  |  |  |  | Noc1: -117dBm or -116.5dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Noc3: = Noc1 +1dB Noc4: = Noc1 +1dB Noc5: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -0.2dB Ês4 / Noc3: +3dB Ês5 / Noc3: -0.2dB Ês6 / Noc4: +3dB Ês7 / Noc4: -0.2dB Ês8 / Noc5: +3dB Ês9 / Noc5: -0.2dB Ês10 / Noc6: +3dB Ês11 / Noc6: -0.2dB Ês12 / Noc6: +3dB Ês13 / Noc6: -0.2dB Cell 1, absolute: RSRP_13 to RSRP_26 RSRP_14 to RSRP_26 RSRP_14 to RSRP_27 RSRP_15 to RSRP_27 RSRP_15 to RSRP_28 RSRP_16 to RSRP_29 RSRP_17 to RSRP_29 depending on operating band Cell 2 and Cell 4, Cell 6, Cell 8, Cell 10, Cell 12 absolute: RSRP_21 to RSRP_34 RSRP_22 to RSRP_34 RSRP_22 to RSRP_35 RSRP_23 to RSRP_35 RSRP_23 to RSRP_36 RSRP_24 to RSRP_37 RSRP_25 to RSRP_37 depending on Cell 1 band Relative: (Cell 2 – Cell 1) and (Cell 4 – Cell 1),(Cell 6 –Cell 1), (Cell 8 – Cell 1), (Cell 10 – Cell 1), (Cell 12 – Cell 1): RSRP_(x+2) to RSRP_(x+15) RSRP_(x-1) to RSRP_(x+17) (Cell 3 - Cell 2) and (Cell 5 - Cell 4), (Cell 7 - Cell 6), (Cell 9 – Cell 8), (Cell 11 – Cell 10), (Cell 13 – Cell 12): RSRP_(x-8) to RSRP_(x+1) The Test Tolerance values are taken from the analysis for the related 3DL Test case 9.1.37, and no separate analysis is needed. |  |  |
| 9.2.1.1 FDD Intra Frequency Absolute RSRQ Accuracy |  |  | Test 1: Noc: -84.76dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB Reported RSRQ values: ±3.5dB Test 3: Noc: -116dBm or -114dBm or -113dBm or -115dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  |  | Test 1: -0.75dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB +0.4dB +0.4dB Via mapping |  |  |  |  |  | Test 1: Noc: -85.51Bm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_04 to RSRQ_16 Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB RSRQ_00 to RSRP_14 Test 3: Noc: -116dBm or -114dBm or -113dBm or -115dBm /15kHz depending on operating band Ês1 / Noc: -3.6dB Ês2 / Noc: -3.6dB RSRQ_00 to RSRQ_14 |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For test 1 the RSRQ values are 1.5dB wider at each end for extreme conditions, and for tests 2 and 3 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.1.1_2 FDD Intra Frequency Absolute RSRQ Accuracy for UE Category 1bis |  |  | Test 1: Noc: -84.76dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±3.5dB Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB Reported RSRQ values: ±4.5dB Test 3: Noc: -116dBm or -114dBm or -113dBm or -115dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±4.5dB |  |  | Test 1: -1.05dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB +0.4dB +0.4dB Via mapping |  |  |  |  |  | Test 1: Noc: -85.81Bm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_02 to RSRQ_18 Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB RSRQ_00 to RSRP_16 Test 3: Noc: -116dBm or -114dBm or -113dBm or -115dBm /15kHz depending on operating band Ês1 / Noc: -3.6dB Ês2 / Noc: -3.6dB RSRQ_00 to RSRQ_16 |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For test 1 the RSRQ values are 1.5dB wider at each end for extreme conditions, and for tests 2 and 3 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.1.1_3 FDD Intra Frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements |  |  | Test 1: Noc: -84.76dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±3.5dB Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB Reported RSRQ values: ±4.5dB Test 3: Noc: -116dBm or -114dBm or -113dBm or -115dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±4.5dB |  |  | Test 1: -1.05dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB +0.4dB +0.4dB Via mapping |  |  |  |  |  | Test 1: Noc: -85.81Bm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_02 to RSRQ_18 Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB RSRQ_00 to RSRP_16 Test 3: Noc: -116dBm or -114dBm or -113dBm or -115dBm /15kHz depending on operating band Ês1 / Noc: -3.6dB Ês2 / Noc: -3.6dB RSRQ_00 to RSRQ_16 |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For test 1 the RSRQ values are 1.5dB wider at each end for extreme conditions, and for tests 2 and 3 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.2.1 TDD Intra Frequency Absolute RSRQ Accuracy |  |  | Test 1: Noc: -84.76dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB Reported RSRQ values: ±3.5dB Test 3: Noc: -116dBm/15kHz Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  |  | Test 1: -0.75dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB +0.4dB +0.4dB Via mapping |  |  |  |  |  | Test 1: Noc: -85.51Bm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_04 to RSRQ_16 Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB RSRQ_00 to RSRQ_14 Test 3: Noc: -116dBm/15kHz Ês1 / Noc: -3.6dB Ês2 / Noc: -3.6dB RSRQ_00 to RSRQ_14 |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For test 1 the RSRQ values are 1.5dB wider at each end for extreme conditions, and for test 2 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.2.1_2 TDD Intra Frequency Absolute RSRQ Accuracy for UE Category 1bis |  |  | Same as 9.2.1.1_2 |  |  | Same as 9.2.1.1_2 |  |  |  |  |  | Same as 9.2.1.1_2 |  |  |
| 9.2.3.1 FDD - FDD Inter Frequency Absolute RSRQ Accuracy |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -80dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB for normal conditions and ±4dB for extreme conditions Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Test 3: Noc1: -119.5dBm to -116dBm/15kHz depending on operating band Noc2: -119.5dBm to -116dBm/15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions |  |  | Test 1: 0dB -1.1dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB 0.3dB 0dB 0.8dB Via mapping |  |  |  |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -81.1dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_04 to RSRQ_16 (NTC) RSRQ_01 to RSRQ_19 (ETC) Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_00 to RSRQ_16 (NTC) RSRQ_00 to RSRQ_17 (ETC) Test 3: Noc1: -119.5dBm to-116 dBm /15kHz depending on operating band Noc2: -119.2dBm -115.7dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_00 to RSRQ_16 (NTC) RSRQ_00 to RSRQ_17 (ETC) |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.3.1_2 FDD - FDD Inter Frequency Absolute RSRQ Accuracy for UE Category 1bis |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -80dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3.5dB for normal conditions and ±5dB for extreme conditions Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported RSRQ values: ±4.5dB for normal conditions and ±5dB for extreme conditions Test 3: Noc1: -119.5dBm to -116dBm/15kHz depending on operating band Noc2: -119.5dBm to -116dBm/15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported RSRQ values: ±4.5dB for normal conditions and ±5dB for extreme conditions |  |  | Test 1: 0dB -1.1dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB 0.3dB 0dB 0.8dB Via mapping |  |  |  |  |  |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -81.1dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_02 to RSRQ_18 (NTC) RSRQ_00 to RSRQ_21 (ETC) Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_00 to RSRQ_18 (NTC) RSRQ_00 to RSRQ_19 (ETC) Test 3: Noc1: -119.5dBm to-116 dBm /15kHz depending on operating band Noc2: -119.2dBm -115.7dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_00 to RSRQ_18 (NTC) RSRQ_00 to RSRQ_19 (ETC) |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.3.1_3 FDD - FDD Inter Frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements for Overlapping Carrier |  |  | Test 1: Noc1: -80 dBm/15kHz Noc2: -80 dBm/15kHz Ês1 / Noc1: -1,75 dB Ês2 / Noc2: -1,75 dB Reported RSRQ values: ±4 dB for normal conditions and ±5.5 dB for extreme conditions Test 2: Noc1: -104.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc1: -4 dB Ês2 / No2c: -4 dB Reported RSRQ values: ±5 dB for normal conditions and ±5.5 dB for extreme conditions Test 3: Noc1: -119.5 dBm to -116 dBm/15kHz depending on operating band Noc2: -119.5 dBm to -116 dBm/15kHz depending on operating band Ês1 / Noc1: -4 dB Ês2 / No2c: -4 dB Reported RSRQ values: ±5 dB for normal conditions and ±5.5 dB for extreme conditions |  |  | Test 1: 0dB -1.1dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB 0.3dB 0dB 0.8dB Via mapping |  |  |  |  |  |  |  | Test 1: Noc1: -80 dBm/15kHz Noc2: -81,1 dBm/15kHz Ês1 / Noc1: -1,75 dB Ês2 / Noc2: -1,75 dB RSRQ 1 to RSRQ 19 (NTC) RSRQ 0 to RSRQ 22 (ETC) Test 2: Noc1: -104.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc1: -4 dB Ês2 / No2c: -3.2 dB RSRQ 0 to RSRQ 19 (NTC) RSRQ 0 to RSRQ 20 (ETC) Test 3: Noc1: -119.5 dBm to -116 dBm /15kHz depending on operating band Noc2: -119.2 dBm to -115.7 dBm /15kHz depending on operating band Ês1 / Noc1: -4 dB Ês2 / No2c: -3.2 dB RSRQ 0 to RSRQ 19 (NTC) RSRQ 0 to RSRQ 20 (ETC) |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.3.1_4 FDD - FDD Inter Frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements for Overlapping Carrier |  |  | Test 1: Noc1: -80 dBm/15kHz Noc2: -80 dBm/15kHz Ês1 / Noc1: -1,75 dB Ês2 / Noc2: -1,75 dB Reported RSRQ values: ±5 dB for normal conditions and ±6.5 dB for extreme conditions Test 2: Noc1: -104.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc1: -4 dB Ês2 / No2c: -4 dB Reported RSRQ values: ±6 dB for normal conditions and ±6.5 dB for extreme conditions Test 3: Noc1: -119.5 dBm to -116 dBm/15kHz depending on operating band Noc2: -119.5 dBm to -116 dBm/15kHz depending on operating band Ês1 / Noc1: -4 dB Ês2 / No2c: -4 dB Reported RSRQ values: ±6 dB for normal conditions and ±6.5 dB for extreme conditions |  |  | Test 1: 0dB -1.1dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB 0.3dB 0dB 0.8dB Via mapping |  |  |  |  |  |  |  | Test 1: Noc1: -80 dBm/15kHz Noc2: -81,1 dBm/15kHz Ês1 / Noc1: -1,75 dB Ês2 / Noc2: -1,75 dB RSRQ 0 to RSRQ 21 (NTC) RSRQ 0 to RSRQ 24 (ETC) Test 2: Noc1: -104.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc1: -4 dB Ês2 / No2c: -3.2 dB RSRQ 0 to RSRQ 21 (NTC) RSRQ 0 to RSRQ 22 (ETC) Test 3: Noc1: -119.5 dBm to -116 dBm /15kHz depending on operating band Noc2: -119.2 dBm to -115.7 dBm /15kHz depending on operating band Ês1 / Noc1: -4 dB Ês2 / No2c: -3.2 dB RSRQ 0 to RSRQ 21 (NTC) RSRQ 0 to RSRQ 22 (ETC) |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.3.2 FDD - FDD Inter Frequency Relative RSRQ Accuracy |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -80dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported Relative RSRQ values: ±3dB for normal conditions and ±4dB for extreme conditions Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported Relative RSRQ values: ±4dB Test 3: Noc1: -119.5dBm to -116dBm/15kHz depending on operating band Noc2: -119.5dBm to -116dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported Relative RSRQ values: ±4dB |  |  | Test 1: -1.1dB -1.1dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB +0.3dB 0dB 0.8dB Via mapping |  |  |  |  |  | Test 1: Noc1: -81.1dBm/15kHz Noc2: -81.1dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x - 8 to RSRQ_x + 8 (NTC) RSRQ_x - 10 to RSRQ_x + 10 (ETC) Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_x - 9 to RSRQ_x + 11 Test 3: Noc1: -119.5dBm to -116dBm/15kHz depending on operating band Noc2: -119.2dBm to -115.7dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_x - 9 to RSRQ_x + 11 |  |  |
|  |  |  | The derivation of the relative RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1 and Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.3.2_2 FDD - FDD Inter Frequency Relative RSRQ Accuracy for UE Category 1bis |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -80dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported Relative RSRQ values: ±4dB for normal conditions and ±4dB for extreme conditions Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported Relative RSRQ values: ±5dB Test 3: Noc1: -119.5dBm to -116dBm/15kHz depending on operating band Noc2: -119.5dBm to -116dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported Relative RSRQ values: ±5dB |  |  | Test 1: -1.1dB -1.1dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB +0.3dB 0dB 0.8dB Via mapping |  |  |  |  |  | Test 1: Noc1: -81.1dBm/15kHz Noc2: -81.1dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x - 10 to RSRQ_x + 10 (NTC) RSRQ_x - 12 to RSRQ_x + 12 (ETC) Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_x - 11 to RSRQ_x + 13 Test 3: Noc1: -119.5dBm to -116dBm/15kHz depending on operating band Noc2: -119.2dBm to -115.7dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_x - 11 to RSRQ_x + 13 |  |  |
|  |  |  | The derivation of the relative RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1 and Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.4.1 TDD - TDD Inter Frequency Absolute RSRQ Accuracy |  |  | Same as 9.2.3.1 |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.4.1_1 TDD - TDD Inter Frequency Absolute RSRQ Accuracy (Rel-12 and forward) |  |  | Same as 9.2.3.1 |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.4.1_2 TDD - TDD Inter Frequency Absolute RSRQ Accuracy for UE Category 1bis |  |  | Same as 9.2.3.1_2 |  |  | Same as 9.2.3.1_2 |  |  |  |  |  | Same as 9.2.3.1_2 |  |  |
| 9.2.4.2 TDD - TDD Inter Frequency Relative RSRQ Accuracy |  |  | Same as 9.2.3.2 |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.4.2_1 TDD - TDD Inter Frequency Relative RSRQ Accuracy (Rel-12 and forward) |  |  | Same as 9.2.3.2 |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.4.2_2 TDD - TDD Inter Frequency Relative RSRQ Accuracy for UE Category 1bis |  |  | Same as 9.2.3.2_2 |  |  | Same as 9.2.3.2_2 |  |  |  |  |  | Same as 9.2.3.2_2 |  |  |
| 9.2.4A.1 FDD - TDD Inter Frequency Absolute RSRQ Accuracy |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -80dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB for normal conditions and ±4dB for extreme conditions Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4.0dB Ês2 / No2c: -4.0dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Test 3: Noc1: -114.5dBm/15kHz Noc2: -114.5dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions |  |  | Test 1: 0dB -1.10dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -81.10dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_04 to RSRQ_16 (NTC) RSRQ_01 to RSRQ_19 (ETC) Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_00 to RSRQ_16 (NTC) RSRQ_00 to RSRQ_17 (ETC) Test 3: Noc1: -114.5dBm/15kHz Noc2: -114.5dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_00 to RSRQ_16 (NTC) RSRQ_00 to RSRQ_17 (ETC) |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.4A.1_2 FDD - TDD Inter Frequency Absolute RSRQ Accuracy for UE Category 1bis |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -80dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3.5dB for normal conditions and ±5dB for extreme conditions Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4.0dB Ês2 / No2c: -4.0dB Reported RSRQ values: ±4.5dB for normal conditions and ±5dB for extreme conditions Test 3: Noc1: -114.5dBm/15kHz Noc2: -114.5dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported RSRQ values: ±4.5dB for normal conditions and ±5dB for extreme conditions |  |  | Test 1: 0dB -1.10dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -81.10dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_02 to RSRQ_18 (NTC) RSRQ_00 to RSRQ_21 (ETC) Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_00 to RSRQ_18 (NTC) RSRQ_00 to RSRQ_19 (ETC) Test 3: Noc1: -114.5dBm/15kHz Noc2: -114.5dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_00 to RSRQ_18 (NTC) RSRQ_00 to RSRQ_19 (ETC) |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.4A.2 FDD - TDD Inter Frequency Relative Accuracy of RSRQ |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -80dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported Relative RSRQ values: ±3dB for normal conditions and ±4dB for extreme conditions Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported Relative RSRQ values: ±4dB Test 3: Noc1: -114.5dBm/15kHz Noc2: -114.5dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported Relative RSRQ values: ±4dB |  |  |  |  | Test 1: -1.10dB -1.10dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  | Test 1: Noc1: -81.10dBm/15kHz Noc2: -81.10dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x - 8 to RSRQ_x + 8 (NTC) RSRQ_x - 10 to RSRQ_x + 10 (ETC) Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_x - 10 to RSRQ_x + 10 Test 3: Noc1: -114.5dBm/15kHz Noc2: -114.5dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_x - 10 to RSRQ_x + 10 |
|  |  |  | The derivation of the relative RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1 and Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.4A.2_2 FDD - TDD Inter Frequency Relative Accuracy of RSRQ for UE Category 1bis |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -80dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported Relative RSRQ values: ±4dB for normal conditions and ±5dB for extreme conditions Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported Relative RSRQ values: ±5dB Test 3: Noc1: -114.5dBm/15kHz Noc2: -114.5dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported Relative RSRQ values: ±5dB |  |  |  |  | Test 1: -1.10dB -1.10dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  | Test 1: Noc1: -81.10dBm/15kHz Noc2: -81.10dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x - 10 to RSRQ_x + 10 (NTC) RSRQ_x - 12 to RSRQ_x + 12 (ETC) Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_x - 12 to RSRQ_x + 12 Test 3: Noc1: -114.5dBm/15kHz Noc2: -114.5dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_x - 12 to RSRQ_x + 12 |
|  |  |  | The derivation of the relative RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1 and Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.5.1 FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation |  |  | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Ês3 / No2c: -4dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions |  |  |  |  | 0dB 0dB 0dB +0.3dB 0dB Via mapping Via mapping |  |  |  |  |  | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -3.7dB Ês3 / No2c: -4dB Cell 1: RSRQ_00 to RSRQ_15 (NTC) RSRQ_00 to RSRQ_16 (ETC) Cell 2: RSRQ_00 to RSRQ_14 (NTC) RSRQ_00 to RSRQ_15 (ETC) |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.5.2 FDD Relative RSRQ Accuracy E-UTRA for Carrier Aggregation |  |  | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Ês3 / No2c: -4dB Reported RSRQ values: ±4dB |  |  |  |  | 0dB 0dB 0dB +0.3dB 0dB Via mapping |  |  |  |  |  | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -3.7dB Ês3 / No2c: -4dB RSRQ_x - 12 to RSRQ_x + 9 (NTC and ETC) |
| 9.2.6.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation |  |  | Same as 9.2.5.1 |  |  |  |  | Same as 9.2.5.1 |  |  |  |  |  | Same as 9.2.5.1 |
| 9.2.6.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation |  |  | Same as 9.2.5.2 |  |  |  |  | Same as 9.2.5.2 |  |  |  |  |  | Same as 9.2.5.2 |
| 9.2.7.1 FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS |  |  | Test 1: Noc: -84.76dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB Reported RSRQ values: ±2.5dB Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB Reported RSRQ values: ±2.5dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  |  |  |  | Test 1: -1.0dB 0dB +0.8dB Via mapping Test 2: 0dB 0dB +0.8dB Via mapping Test 3: 0dB 0dB +0.95dB Via mapping |  |  |  |  |  | Test 1: Noc: -85.76Bm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -1.2dB RSRQ_04 to RSRQ_16 Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -1.2dB RSRQ_04 to RSRQ_16 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRQ_00 to RSRQ_16 |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For tests 1 and 2 the RSRQ values are 1.5dB wider at each end for extreme conditions, and for test 3 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.8.1 TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS |  |  | Subset of 9.2.7.1 |  |  |  |  | Same as 9.2.7.1 |  |  |  |  |  | Subset of 9.2.7.1 |
| 9.2.9.1 FDD Absolute RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) |  |  | Test 1: Noc: -84.76dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB Reported RSRQ values: ±2.5dB Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB Reported RSRQ values: ±2.5dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  |  |  |  | Test 1: -1.0dB 0dB +0.8dB Via mapping Test 2: 0dB 0dB +0.8dB Via mapping Test 3: 0dB 0dB +0.95dB Via mapping |  |  |  |  |  | Test 1: Noc: -85.76Bm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -1.2dB RSRQ_04 to RSRQ_16 Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -1.2dB RSRQ_04 to RSRQ_16 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRQ_00 to RSRQ_16 |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For tests 1 and 2 the RSRQ values are 1.5dB wider at each end for extreme conditions, and for test 3 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.10.1 TDD Absolute RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) |  |  | Subset of 9.2.9.1 |  |  |  |  | Same as 9.2.9.1 |  |  |  |  |  | Subset of 9.2.9.1 |
| 9.2.11.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz |  |  | Same as 9.2.5.1 |  |  |  |  | Same as 9.2.5.1 |  |  |  |  |  | Same as 9.2.5.1 |
| 9.2.11.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz |  |  | Same as 9.2.5.2 |  |  |  |  | Same as 9.2.5.2 |  |  |  |  |  | Same as 9.2.5.2 |
| 9.2.12.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz |  |  | Same as 9.2.5.1 |  |  |  |  | Same as 9.2.5.1 |  |  |  |  |  | Same as 9.2.5.1 |
| 9.2.12.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz |  |  | Same as 9.2.5.2 |  |  |  |  | Same as 9.2.5.2 |  |  |  |  |  | Same as 9.2.5.2 |
| 9.2.15.1 FDD RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) |  |  | Test 1: Noc: -84.76dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -1.5dB Reported RSRQ values: ±2.5dB Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -1.5dB Reported RSRQ values: ±2.5dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  |  | Test 1: 0dB 0dB 0dB +1.0dB Via mapping Test 2: 0dB 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB 0dB +1.0dB Via mapping |  |  |  |  |  | Test 1: Noc: -84.76Bm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -0.5dB RSRQ_05 to RSRQ_17 Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -0.5dB RSRQ_05 to RSRQ_17 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -3.0dB RSRQ_00 to RSRQ_16 |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 3 RSRQ from Ês1 / Noc, Ês2 / Noc, and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For tests 1 and 2 the RSRQ values are 1.5dB wider at each end for extreme conditions, and for test 3 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.16.1 TDD RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) |  |  | Subset of 9.2.15.1 |  |  | Same as 9.2.15.1 |  |  |  |  |  | Subset of 9.2.15.1 |  |  |
| 9.2.17.1 FDD Intra Frequency Absolute RSRQ Accuracy for 5MHz Bandwidth |  |  | Test 1: Noc: -81.76dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: Noc: -100.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB Reported RSRQ values: ±3.5dB Test 3: Noc: -109.5dBm /15kHz Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  |  | Test 1: -1.05dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB +0.4dB +0.4dB Via mapping |  |  |  |  |  | Test 1: Noc: -82.81Bm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_04 to RSRQ_16 Test 2: Noc: -100.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB RSRQ_00 to RSRP_14 Test 3: Noc: -109.5dBm /15kHz Ês1 / Noc: -3.6dB Ês2 / Noc: -3.6dB RSRQ_00 to RSRQ_14 |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For test 1 the RSRQ values are 1.5dB wider at each end for extreme conditions, and for tests 2 and 3 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.18.1 FDD - FDD Inter Frequency Absolute RSRQ Accuracy for 5MHz Bandwidth |  |  | Test 1: Noc1: -77dBm/15kHz Noc2: -77dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB for normal conditions and ±4dB for extreme conditions Test 2: Noc1: -101.7dBm/15kHz Noc2: -101.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Test 3: Noc1: -119.5dBm to -116dBm/15kHz depending on operating band Noc2: 113dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions |  |  | Test 1: 0dB -1.1dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB 0.3dB 0dB 0.8dB Via mapping |  |  |  |  |  | Test 1: Noc1: -77dBm/15kHz Noc2: -78.1dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_04 to RSRQ_16 (NTC) RSRQ_01 to RSRQ_19 (ETC) Test 2: Noc1: -101.7dBm/15kHz Noc2: -101.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_00 to RSRQ_16 (NTC) RSRQ_00 to RSRQ_17 (ETC) Test 3: Noc1: -119.5dBm to -116dBm /15kHz depending on operating band Noc2: -112.7dBm /15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_00 to RSRQ_16 (NTC) RSRQ_00 to RSRQ_17 (ETC) |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.18.2 FDD - FDD Inter Frequency Relative RSRQ Accuracy for 5MHz Bandwidth |  |  | Test 1: Noc1: -77dBm/15kHz Noc2: -77dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported Relative RSRQ values: ±3dB for normal conditions and ±4dB for extreme conditions Test 2: Noc1: -101.7dBm/15kHz Noc2: -101.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported Relative RSRQ values: ±4dB Test 3: Noc1: -119.5dBm to -116dBm/15kHz depending on operating band Noc2: 113dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported Relative RSRQ values: ±4dB |  |  |  | Test 1: -1.1dB -1.1dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB +0.3dB 0dB 0.8dB Via mapping |  |  |  | Test 1: Noc1: -78.1dBm/15kHz Noc2: -78.1dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x - 8 to RSRQ_x + 8 (NTC) RSRQ_x - 10 to RSRQ_x + 10 (ETC) Test 2: Noc1: -101.7dBm/15kHz Noc2: -101.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_x - 9 to RSRQ_x + 11 Test 3: Noc1: -119.5dBm to -116dBm/15kHz depending on operating band Noc2: -112.7dBm /15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_x - 9 to RSRQ_x + 11 |  |  |  |
|  |  |  | The derivation of the relative RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1 and Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.19.1 FDD-FDD Inter Frequency absolute WB-RSRQ accuracy |  |  | FFS |  |  |  | FFS |  |  |  | FFS |  |  |  |
| 9.2.20.1 TDD-TDD Inter Frequency absolute WB-RSRQ accuracy |  |  | FFS |  |  |  | FFS |  |  |  | FFS |  |  |  |
| 9.2.21.1 FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz |  |  | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Ês3 / No2c: -4dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions |  |  |  | 0dB 0dB 0dB +0.3dB 0dB Via mapping Via mapping |  |  |  |  | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -3.7dB Ês3 / No2c: -4dB Cell 1: RSRQ_00 to RSRQ_15 (NTC) RSRQ_00 to RSRQ_16 (ETC) Cell 2: RSRQ_00 to RSRQ_14 (NTC) RSRQ_00 to RSRQ_15 (ETC) |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.21.2 FDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz |  |  | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Ês3 / No2c: -4dB Reported RSRQ values: ±4dB |  |  |  | 0dB 0dB 0dB +0.3dB 0dB Via mapping |  |  |  |  | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -3.7dB Ês3 / No2c: -4dB RSRQ_x - 12 to RSRQ_x + 9 (NTC and ETC) |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.22.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz |  |  | Same as 9.2.21.1 |  |  |  | Same as 9.2.21.1 |  |  |  |  | Same as 9.2.21.1 |  |  |
| 9.2.22.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz |  |  | Same as 9.2.21.2 |  |  |  | Same as 9.2.21.2 |  |  |  |  | Same as 9.2.21.2 |  |  |
| 9.2.23.1 FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz |  |  | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB Ês3 / Noc2: -4dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions |  |  |  | 0dB 0dB 0dB +0.3dB 0dB Via mapping Via mapping |  |  |  |  | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / Noc2: -3.7dB Ês3 / Noc2: -4dB Cell 1: RSRQ_00 to RSRQ_15 (NTC) RSRQ_00 to RSRQ_16 (ETC) Cell 2: RSRQ_00 to RSRQ_14 (NTC) RSRQ_00 to RSRQ_15 (ETC) |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.23.2 FDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz |  |  | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB Ês3 / Noc2: -4dB Reported RSRQ values: ±4dB |  |  |  | 0dB 0dB 0dB +0.3dB 0dB Via mapping |  |  |  |  | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / Noc2: -3.7dB Ês3 / Noc2: -4dB RSRQ_x - 12 to RSRQ_x + 9 (NTC and ETC) |  |  |
| 9.2.24.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz |  |  | Same as 9.2.23.1 |  |  |  | Same as 9.2.23.1 |  |  |  |  | Same as 9.2.23.1 |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.24.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz |  |  | Same as 9.2.23.2 |  |  |  | Same as 9.2.23.2 |  |  |  |  | Same as 9.2.23.2 |  |  |
| 9.2.25.1 Absolute RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD |  |  | Same as 9.2.38 Cell 1 and Cell 2, absolute RSRQ part |  |  |  | Same as 9.2.38 Cell 1 and Cell 2 |  |  |  |  | Same as 9.2.38 Cell 1 and Cell 2, absolute RSRQ part |  |  |
| 9.2.25.2 Relative RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD |  |  | Same as 9.2.38 Cell 1 and Cell 2, absolute RSRQ part |  |  |  | Same as 9.2.38 Cell 1 and Cell 2 |  |  |  |  | Same as 9.2.38 Cell 1 and Cell 2, absolute RSRQ part |  |  |
| 9.2.26.1 Absolute RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD |  |  | Same as 9.2.39 Cell 1 and Cell 2, absolute RSRQ part |  |  |  | Same as 9.2.39 Cell 1 and Cell 2 |  |  |  |  | Same as 9.2.39 Cell 1 and Cell 2, absolute RSRQ part |  |  |
| 9.2.26.2 Relative RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD |  |  | Same as 9.2.39 Cell 1 and Cell 2, absolute RSRQ part |  |  |  | Same as 9.2.39 Cell 1 and Cell 2 |  |  |  |  | Same as 9.2.39 Cell 1 and Cell 2, absolute RSRQ part |  |  |
| 9.2.27.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz+10MHz |  |  | Noc1: -119.5dBm or -118.5dBm or -117.5 dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114 dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Ês3 / No2c: -4dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions |  |  |  | 0dB 0dB 0dB +0.3dB 0dB Via mapping Via mapping |  |  |  |  | Noc1: -119.5dBm or -118.5dBm or -117.5 dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114 dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -3.7dB Ês3 / No2c: -4dB Cell 1: RSRQ_00 to RSRQ_15 (NTC) RSRQ_00 to RSRQ_16 (ETC) Cell 2: RSRQ_00 to RSRQ_14 (NTC) RSRQ_00 to RSRQ_15 (ETC) |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.27.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz+10MHz |  |  | Noc1: -119.5dBm or -118.5dBm or -117.5 dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114 dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Ês3 / No2c: -4dB Reported RSRQ values: ±4dB |  |  |  | 0dB 0dB 0dB +0.3dB 0dB Via mapping |  |  |  |  | Noc1: -119.5dBm or -118.5dBm or -117.5 dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114 dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -3.7dB Ês3 / No2c: -4dB RSRQ_x - 12 to RSRQ_x + 9 (NTC and ETC) |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.28 FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal |  |  | Test 1: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  |  |  | Test 1: 0dB +0.4dB +0.4dB Via mapping |  |  |  |  | Test 1: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: -3.6dB Ês2 / Noc: -3.6dB RSRQ_00 to RSRQ_14 |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For test 1 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.29 TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal |  |  | Test 1: Noc: -116dBm to -114dBm /15kHz Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  |  |  | Test 1: 0dB +0.4dB +0.4dB Via mapping |  |  |  |  | Test 1: Noc: -116dBm to -114dBm /15kHz Ês1 / Noc: -3.6dB Ês2 / Noc: -3.6dB RSRQ_00 to RSRQ_14 |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For test 1 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.38 3DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation |  |  | Noc1: -119.5dBm to -116dBm /15kHz depending on operating band Noc2: -116dBm to -114dBm /15kHz depending on operating band Noc3: -116dBm to -114dBm /15kHz depending on operating band Ês1 / Noc1: -6dB Ês2 / Noc2: -6dB Ês3 / Noc3: -6dB Reported absolute RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Reported relative RSRQ values: (Cell 2 - Cell 1) and (Cell 3 - Cell 1): ±4dB for normal conditions ±4dB for extreme conditions |  |  |  | 0.3dB 0dB 0dB +0.8dB +0.8dB +0.8dB Via mapping Via mapping |  |  |  |  | Noc1: -119.2dBm to -115.7dBm /15kHz depending on operating band Noc2: -116dBm to -114dBm /15kHz depending on operating band Noc3: -116dBm to -114dBm /15kHz depending on operating band Ês1 / Noc1: -5.20dB Ês2 / Noc2: -5.20dB Ês3 / Noc3: -5.20dB Absolute: Cell 1, Cell 2, and Cell 3 RSRQ_0 to RSRQ_14 Relative: (Cell 2 - Cell 1) and (Cell 3 - Cell 1): RSRQ_(x-10) to RSRQ_(x+10) |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty from all applicable Noc and Ês / Noc values, the allowed UE reporting accuracy, and the UE mapping function. Where extreme condition UE accuracies differ from normal condition accuracies, reported values are recalculated. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.30 FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal |  |  | Test 1: Noc1: -117.5dBm to -114dBm/15kHz depending on operating band Noc2: -117.5dBm to -114dBm/15kHz depending on operating band Ês1 / Noc1: -6dB Ês2 / No2c: -6dB Reported absolute RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Reported absolute RSRQ values: ±4dB for both normal conditions and extreme conditions |  |  |  | Test 1: 0dB 0.3dB 0.8dB 0.8dB Via mapping |  |  |  |  | Test 1: Noc1: -117.5dBm to-114 dBm /15kHz depending on operating band Noc2: -117.2dBm -113.7dBm /15kHz depending on operating band Ês1 / Noc1: -5.2dB Ês2 / No2c: -5.2dB RSRQ_00 to RSRQ_14 (NTC) RSRQ_00 to RSRQ_15 (ETC) RSRQ_x - 10 to RSRQ_x + 10 |  |  |
|  |  |  | The derivation of the absolute RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The derivation of the relative RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1 and Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.31 TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal |  |  | Test 1: Noc1: -117.5dBm to -115.5dBm/15kHz depending on operating band Noc2: -117.5dBm to -115.5dBm/15kHz depending on operating band Ês1 / Noc1: -6dB Ês2 / No2c: -6dB Reported absolute RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Reported absolute RSRQ values: ±4dB for both normal conditions and extreme conditions |  |  |  | Test 1: 0dB 0.3dB 0.8dB 0.8dB Via mapping |  |  |  |  | Test 1: Noc1: -117.5dBm to-115.5 dBm /15kHz depending on operating band Noc2: -117.2dBm -115.2dBm /15kHz depending on operating band Ês1 / Noc1: -5.2dB Ês2 / No2c: -5.2dB RSRQ_00 to RSRQ_14 (NTC) RSRQ_00 to RSRQ_15 (ETC) RSRQ_x - 10 to RSRQ_x + 10 |  |  |
|  |  |  | The derivation of the absolute RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The derivation of the relative RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1 and Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.39 3DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation |  |  | Same as 9.2.38 |  |  |  | Same as 9.2.38 |  |  |  |  | Same as 9.2.38 |  |  |
| 9.2.40 3DL FDD RSRQ for E-UTRA Carrier Aggregation |  |  | Same as 9.2.38 |  |  |  | Same as 9.2.38 |  |  |  |  | Same as 9.2.38 |  |  |
| 9.2.41 3DL TDD RSRQ for E-UTRA Carrier Aggregation |  |  | Same as 9.2.38 |  |  |  | Same as 9.2.38 |  |  |  |  | Same as 9.2.38 |  |  |
| 9.2.42.1 FD-FDD Intra Frequency Absolute RSRQ Accuracy for UE category 0 |  |  | Test 1: Noc: -84.76dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±3.5dB Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB Reported RSRQ values: ±4.5dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±4.5dB |  |  |  | Test 1: -1.05dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB +0.4dB +0.4dB Via mapping |  |  |  |  | Test 1: Noc: -85.81Bm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_02 to RSRQ_18 Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB RSRQ_00 to RSRP_16 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: -3.6dB Ês2 / Noc: -3.6dB RSRQ_00 to RSRQ_16 |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For test 1 the RSRQ values are 1.5dB wider at each end for extreme conditions, and for tests 2 and 3 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.43.1 HD-FDD Intra Frequency Absolute RSRQ Accuracy for UE category 0 |  |  | Same as 9.2.42.1 |  |  |  | Same as 9.2.42.1 |  |  |  |  | Same as 9.2.42.1 |  |  |
| 9.2.44.1 TDD Intra Frequency Absolute RSRQ Accuracy for UE category 0 |  |  | Test 1: Noc: -84.76dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±3.5dB Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB Reported RSRQ values: ±4.5dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±4.5dB |  |  |  | Test 1: -1.05dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB +0.4dB +0.4dB Via mapping |  |  |  |  | Test 1: Noc: -85.81Bm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_02 to RSRQ_18 Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB RSRQ_00 to RSRP_16 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: -3.6dB Ês2 / Noc: -3.6dB RSRQ_00 to RSRQ_16 |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For test 1 the RSRQ values are 1.5dB wider at each end for extreme conditions, and for test 2 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.45 4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation |  |  | Noc1: -119.5dBm to -116dBm /15kHz depending on operating band Noc2: -116dBm to -114dBm /15kHz depending on operating band Noc3: -116dBm to -114dBm /15kHz depending on operating band Noc4: -116dBm to -114dBm /15kHz depending on operating band Ês1 / Noc1: -6dB Ês2 / Noc2: -6dB Ês3 / Noc3: -6dB Ês4 / Noc4: -6dB Reported absolute RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Reported relative RSRQ values: (Cell 2 - Cell 1) , (Cell 3 - Cell 1) and (Cell 4 - Cell 1) : ±4dB for normal conditions ±4dB for extreme conditions |  |  |  | 0.3dB 0dB 0dB 0dB +0.8dB +0.8dB +0.8dB +0.8dB Via mapping Via mapping |  |  |  |  | Noc1: -119.2dBm to -115.7dBm /15kHz depending on operating band Noc2: -116dBm to -114dBm /15kHz depending on operating band Noc3: -116dBm to -114dBm /15kHz depending on operating band Noc4: -116dBm to -114dBm /15kHz depending on operating band Ês1 / Noc1: -5.20dB Ês2 / Noc2: -5.20dB Ês3 / Noc3: -5.20dB Ês3 / Noc3: -5.20dB Absolute: Cell 1, Cell 2, Cell 3 and Cell 4 RSRQ_00 to RSRQ_14 (NC) RSRQ_00 to RSRQ_15 (EC) Relative: (Cell 2 - Cell 1) , (Cell 3 - Cell 1) and (Cell 4 - Cell 1): RSRQ_(x-10) to RSRQ_(x+10) The Test Tolerance values are taken from the analysis for the related 3DL Test case 9.2.38, and no separate analysis is needed |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty from all applicable Noc and Ês / Noc values, the allowed UE reporting accuracy, and the UE mapping function. Where extreme condition UE accuracies differ from normal condition accuracies, reported values are recalculated. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.46 4DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation |  |  | Same as 9.2.45 |  |  |  | Same as 9.2.45 |  |  |  |  | Same as 9.2.45 |  |  |
| 9.2.47 5 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation |  |  | Noc1: -119.5dBm to -116dBm /15kHz depending on operating band Noc2: -116dBm to -114dBm /15kHz depending on operating band Noc3: -116dBm to -114dBm /15kHz depending on operating band Noc4: -116dBm to -114dBm /15kHz depending on operating band Noc5: -116dBm to -114dBm /15kHz depending on operating band Ês1 / Noc1: -6dB Ês2 / Noc2: -6dB Ês3 / Noc3: -6dB Ês4 / Noc4: -6dB Ês5 / Noc5: -6dB Reported absolute RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Reported relative RSRQ values: (Cell 2 - Cell 1) , (Cell 3 - Cell 1) , (Cell 4 - Cell 1) and (Cell 5 - Cell 1) : ±4dB for normal conditions ±4dB for extreme conditions |  |  |  | 0.3dB 0dB 0dB 0dB 0dB +0.8dB +0.8dB +0.8dB +0.8dB +0.8dB Via mapping Via mapping |  |  |  |  | Noc1: -119.2dBm to -115.7dBm /15kHz depending on operating band Noc2: -116dBm to -114dBm /15kHz depending on operating band Noc3: -116dBm to -114dBm /15kHz depending on operating band Noc4: -116dBm to -114dBm /15kHz depending on operating band Noc5: -116dBm to -114dBm /15kHz depending on operating band Ês1 / Noc1: -5.20dB Ês2 / Noc2: -5.20dB Ês3 / Noc3: -5.20dB Ês4 / Noc4: -5.20dB Ês5 / Noc5: -5.20dB Absolute: Cell 1, Cell 2, Cell 3, Cell 4 and Cell 5 RSRQ_00 to RSRQ_14 (NC) RSRQ_00 to RSRQ_15 (EC) Relative: (Cell 2 - Cell 1) , (Cell 3 - Cell 1) , (Cell 4 - Cell 1) and (Cell 5 - Cell 1): RSRQ_(x-10) to RSRQ_(x+10) The Test Tolerance values are taken from the analysis for the related 4DL Test case 9.2.45, and no separate analysis is needed |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty from all applicable Noc and Ês / Noc values, the allowed UE reporting accuracy, and the UE mapping function. Where extreme condition UE accuracies differ from normal condition accuracies, reported values are recalculated. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.48 5 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation |  |  | Same as 9.2.47 |  |  |  | Same as 9.2.47 |  |  |  |  | Same as 9.2.47 |  |  |
| 9.2.51 FDD intra frequency absolute and relative RSRQ accuracies for SCell with frame structure 3 |  |  | Noc1: -119dBm to -116dBm/15kHz depending on operating band Noc2 and Noc3: Noc1 +3.5dB Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB Ês3 / Noc2: -4dB Reported absolute RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Reported absolute RSRQ values: ±4dB for both normal conditions and extreme conditions |  |  |  | 0dB 0dB 0dB 0dB 0.3dB Via mapping |  |  |  |  | Noc1: -119dBm to -116dBm/15kHz depending on operating band Noc2 and Noc3: Noc1 +3.5dB Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB Ês3 / Noc2: -3.7dB RSRQ_00 to RSRQ_14 (NTC) RSRQ_00 to RSRQ_15 (ETC) RSRQ_x - 12 to RSRQ_x + 9 |  |  |
| 9.2.52 TDD intra frequency absolute and relative RSRQ accuracies for SCell with frame structure 3 |  |  | Same as 9.2.51 |  |  |  | Same as 9.2.51 |  |  |  |  | Same as 9.2.51 |  |  |
| 9.2.55 3DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes |  |  | Same as 9.2.38 |  |  |  | Same as 9.2.38 |  |  |  |  | Same as 9.2.38 |  |  |
| 9.2.56 4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes |  |  | Same as 9.2.45 |  |  |  | Same as 9.2.45 |  |  |  |  | Same as 9.2.45 |  |  |
| 9.2.57 5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes |  |  | Same as 9.2.47 |  |  |  | Same as 9.2.47 |  |  |  |  | Same as 9.2.47 |  |  |
| 9.2.58 6 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes |  |  | Noc1: -119.5dBm to -116dBm /15kHz depending on operating band Noc2: -116dBm to -114dBm /15kHz depending on operating band Noc3: -116dBm to -114dBm /15kHz depending on operating band Noc4: -116dBm to -114dBm /15kHz depending on operating band Noc5: -116dBm to -114dBm /15kHz depending on operating band Noc6: -116dBm to -114dBm /15kHz depending on operating band Ês1 / Noc1: -6dB Ês2 / Noc2: -6dB Ês3 / Noc3: -6dB Ês4 / Noc4: -6dB Ês5 / Noc5: -6dB Ês6 / Noc6: -6dB Reported absolute RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Reported relative RSRQ values: (Cell 2 – Cell 1) , (Cell 3 – Cell 1) , (Cell 4 – Cell 1) and (Cell 5 – Cell 1), (Cell 6 – Cell 1): ±4dB for normal conditions ±4dB for extreme conditions |  |  |  | 0.3dB 0dB 0dB 0dB 0dB 0dB +0.8dB +0.8dB +0.8dB +0.8dB +0.8dB +0.8dB Via mapping Via mapping |  |  |  |  | Noc1: -119.2dBm to -115.7dBm /15kHz depending on operating band Noc2: -116dBm to -114dBm /15kHz depending on operating band Noc3: -116dBm to -114dBm /15kHz depending on operating band Noc4: -116dBm to -114dBm /15kHz depending on operating band Noc5: -116dBm to -114dBm /15kHz depending on operating band Noc6: -116dBm to -114dBm /15kHz depending on operating band Ês1 / Noc1: -5.20dB Ês2 / Noc2: -5.20dB Ês3 / Noc3: -5.20dB Ês4 / Noc4: -5.20dB Ês5 / Noc5: -5.20dB Ês6 / Noc6: -5.20dB Absolute: Cell 1, Cell 2, Cell 3, Cell 4 and Cell 5 RSRQ_00 to RSRQ_14 (NC) RSRQ_00 to RSRQ_15 (EC) Relative: (Cell 2 – Cell 1) , (Cell 3 – Cell 1) , (Cell 4 – Cell 1) and (Cell 5 – Cell 1), (Cell 6 – Cell 1): RSRQ_(x-10) to RSRQ_(x+10) The Test Tolerance values are taken from the analysis for the related 4DL Test case 9.2.45, and no separate analysis is needed |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty from all applicable Noc and Ês / Noc values, the allowed UE reporting accuracy, and the UE mapping function. Where extreme condition UE accuracies differ from normal condition accuracies, reported values are recalculated. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.59 7 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes |  |  | Noc1: -119.5dBm to -116dBm /15kHz depending on operating band Noc2: -116dBm to -114dBm /15kHz depending on operating band Noc3: -116dBm to -114dBm /15kHz depending on operating band Noc4: -116dBm to -114dBm /15kHz depending on operating band Noc5: -116dBm to -114dBm /15kHz depending on operating band Noc6: -116dBm to -114dBm /15kHz depending on operating band Noc7: -116dBm to -114dBm /15kHz depending on operating band Ês1 / Noc1: -6dB Ês2 / Noc2: -6dB Ês3 / Noc3: -6dB Ês4 / Noc4: -6dB Ês5 / Noc5: -6dB Ês6 / Noc6: -6dB Ês7 / Noc7: -6dB Reported absolute RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Reported relative RSRQ values: (Cell 2 – Cell 1) , (Cell 3 – Cell 1) , (Cell 4 – Cell 1) and (Cell 5 – Cell 1), (Cell 6 – Cell 1), (Cell 7 – Cell 1): ±4dB for normal conditions ±4dB for extreme conditions |  |  |  | 0.3dB 0dB 0dB 0dB 0dB 0dB 0dB +0.8dB +0.8dB +0.8dB +0.8dB +0.8dB +0.8dB +0.8dB Via mapping Via mapping |  |  |  |  | Noc1: -119.2dBm to -115.7dBm /15kHz depending on operating band Noc2: -116dBm to -114dBm /15kHz depending on operating band Noc3: -116dBm to -114dBm /15kHz depending on operating band Noc4: -116dBm to -114dBm /15kHz depending on operating band Noc5: -116dBm to -114dBm /15kHz depending on operating band Noc6: -116dBm to -114dBm /15kHz depending on operating band Noc7: -116dBm to -114dBm /15kHz depending on operating band Ês1 / Noc1: -5.20dB Ês2 / Noc2: -5.20dB Ês3 / Noc3: -5.20dB Ês4 / Noc4: -5.20dB Ês5 / Noc5: -5.20dB Ês6 / Noc6: -5.20dB Ês7 / Noc7: -5.20dB Absolute: Cell 1, Cell 2, Cell 3, Cell 4 and Cell 5 RSRQ_00 to RSRQ_14 (NC) RSRQ_00 to RSRQ_15 (EC) Relative: (Cell 2 – Cell 1) , (Cell 3 – Cell 1) , (Cell 4 – Cell 1) and (Cell 5 – Cell 1), (Cell 6 – Cell 1), (Cell 7 – Cell 1): RSRQ_(x-10) to RSRQ_(x+10) The Test Tolerance values are taken from the analysis for the related 4DL Test case 9.2.45, and no separate analysis is needed |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty from all applicable Noc and Ês / Noc values, the allowed UE reporting accuracy, and the UE mapping function. Where extreme condition UE accuracies differ from normal condition accuracies, reported values are recalculated. |  |  |  |  |  |  |  |  |  |  |  |
| 9.3.1 E-UTRAN FDD - UTRA FDD CPICH RSCP absolute accuracy |  |  | Test 1: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -60.00dBm/3.84MHz Ior / Ioc: +9.54dB CPICH_Ec/Ior: -10.00dB Reported CPICH_RSCP values: ±8dB Test 2: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -94.46dBm or -92.46dBm or -90.96dBm or -91.46dBm or -93.46dBm /3.84MHz depending on operating band Ior / Ioc: -9.54dB CPICH_Ec/Ior: -10.00dB Reported CPICH_RSCP values: ±6dB |  |  |  | Test 1: 0dB 0dB -0.75dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0.7dB 0.35dB 0dB Via mapping |  |  |  |  | Test 1: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -60.75dBm/3.84MHz Ior / Ioc: +9.54dB CPICH_Ec/Ior: -10.00dB CPICH_RSCP_46 to CPICH_RSCP_63 Test 2: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -93.76dBm or -91.76dBm or -90.26dBm or -90.76dBm or -92.76dBm /3.84MHz depending on operating band Ior / Ioc: -9.19dB CPICH_Ec/Ior: -10.00dB CPICH_RSCP_-04 to CPICH_RSCP_9 CPICH_RSCP_-02 to CPICH_RSCP_11 CPICH_RSCP_-01 to CPICH_RSCP_12 CPICH_RSCP_-03 to CPICH_RSCP_10 depending on operating band |  |  |
|  |  |  | The derivation of the CPICH_RSCP values takes into account the uncertainty in Cell 2 CPICH_RSCP from Ioc, Ior / Ioc and CPICH_Ec/Ior, the allowed UE reporting accuracy, and the UE mapping function. The CPICH_RSCP values given above are for normal conditions. In all cases the CPICH_RSCP values are 3dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.3.2 E-UTRAN TDD - UTRA FDD CPICH RSCP absolute accuracy |  |  | Same as 9.3.1 |  |  |  | Same as 9.3.1 |  |  |  |  | Same as 9.3.1 |  |  |
| 9.3.3 E UTRAN FDD UTRA FDD CPICH RSCP absolute accuracy for 5MHz bandwidth |  |  | Same as 9.3.1 |  |  |  | Same as 9.3.1 |  |  |  |  | Same as 9.3.1 |  |  |
| 9.4.1 E-UTRAN FDD - UTRA FDD CPICH Ec/No absolute accuracy |  |  | Test 1: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -52.22dBm/3.84MHz Ior / Ioc: -1.75dB CPICH_Ec/Ior: -10.00dB Reported CPICH_Ec/Io accuracy values: ±1.5dB for normal conditions and ±3dB for extreme conditions Test 2: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -87.27dBm/3.84MHz Ior / Ioc: -4.7dB CPICH_Ec/Ior: -10.00dB Reported CPICH_Ec/Io accuracy values: ±2dB for normal conditions and ±3dB for extreme conditions Test 3: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -94.46dBm or -92.46dBm or 90.96dBm or -91.46dBm or -93.46dBm/3.84MHz depending on operating band Ior / Ioc: -9.54dB CPICH_Ec/Ior: -10.00dB Reported CPICH_Ec/Io accuracy values: -4.2 to +3dB for normal conditions and extreme conditions. The additional 1.2 dB relaxation to the lower limit for both normal and extreme condition is to take into account the effect of thermal noise and noise generated in the receiver as specified in A.9.4.1.3 of TS 36.133 [4] |  |  |  | Test 1: 0dB -0dB -0.9dB 0.3dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.3dB 0dB Via mapping Test 3: 0dB 0dB 0.7dB 0.4dB 0dB Via mapping |  |  |  |  | Test 1: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -53.12dBm/3.84MHz Ior / Ioc: -1.45dB CPICH_Ec/Ior: -10.00dB CPICH_Ec/Io_17 to CPICH_Ec/Io_24 for normal conditions. CPICH_Ec/Io_14 to CPICH_Ec/Io_27 for extreme conditions Test 2: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -87.27dBm/3.84MHz Ior / Ioc: -4.4dB CPICH_Ec/Ior: -10.00dB CPICH_Ec/Io_13 to CPICH_Ec/Io_22 for normal conditions. CPICH_Ec/Io_11 to CPICH_Ec/Io_24 for extreme conditions Test 3: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -93.76dBm or -91.76dBm or 90.26dBm or -90.76dBm or -92.76dBm /3.84MHz depending on operating band Ior / Ioc: -9.14dB CPICH_Ec/Ior: -10.00dB CPICH_Ec/Io_0 to CPICH_Ec/Io_16 for normal and extreme conditions. |  |  |
| 9.4.2 E-UTRAN TDD - UTRA FDD CPICH Ec/No absolute accuracy |  |  | Same as 9.4.1 |  |  |  | Same as 9.4.1 |  |  |  |  | Same as 9.4.1 |  |  |
| 9.4.3 E-UTRAN FDD - UTRA FDD CPICH Ec/No absolute accuracy for 5MHz bandwidth |  |  | Same as 9.4.1 |  |  |  | Same as 9.4.1 |  |  |  |  | Same as 9.4.1 |  |  |
| 9.5.1 E-UTRAN FDD - UTRA TDD P-CCPCH RSCP absolute accuracy |  |  | Test 1: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -54.1dBm/1.28MHz Ior / Ioc: 2.0dB PCCPCH_EC/Ior: -3dB DwPCH_EC/Ior: 0dB Reported PCCPCH RSCP accuracy values: ±8dB for normal conditions and ±11dB for extreme conditions Test 2: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -75.2dBm/1.28MHz Ior / Ioc: 5.0dB PCCPCH_EC/Ior: -3dB DwPCH_EC/Ior: 0dB Reported PCCPCH RSCP accuracy values: ±8dB for normal conditions and ±11dB for extreme conditions Test 3: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -97.0dBm/1.28MHz Ior / Ioc: 0dB PCCPCH_EC/Ior: -3dB DwPCH_EC/Ior: 0dB Reported PCCPCH RSCP accuracy values: ±6dB for normal conditions and ±9dB for extreme conditions |  |  |  | Test 1: 0dB 0dB -0.8dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0dB 0dB 0dB Via mapping |  |  |  |  | Test 1: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -54.9dBm/1.28MHz Ior / Ioc: 2.0dB PCCPCH_EC/Ior: -3dB DwPCH_EC/Ior: 0dB PCCPCH RSCP 51 to PCCPCH RSCP 68 for normal conditions. PCCPCH RSCP 48 to PCCPCH RSCP 71 for extreme conditions Test 2: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -75.2dBm/1.28MHz Ior / Ioc: 5.0dB PCCPCH_EC/Ior: -3dB DwPCH_EC/Ior: 0dB PCCPCH RSCP 34 to PCCPCH RSCP 51 for normal conditions. PCCPCH RSCP 31 to PCCPCH RSCP 54 for extreme conditions Test 3: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -96.2dBm/1.28MHz Ior / Ioc: 0dB PCCPCH_EC/Ior: -3dB DwPCH_EC/Ior: 0dB PCCPCH RSCP 10 to PCCPCH RSCP 23 for normal conditions. PCCPCH RSCP 07 to PCCPCH RSCP 26 for extreme conditions |  |  |
| 9.5.2 E-UTRAN TDD - UTRA TDD P-CCPCH RSCP absolute accuracy |  |  | Same as 9.5.1 |  |  |  | Same as 9.5.1 |  |  |  |  | Same as 9.5.1 |  |  |
| 9.6.1 GSM RSSI accuracy for E-UTRAN FDD |  |  | Subtest 1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -38.5dBm GSM Cell 3 Signal level: -38.5dBm Subtest 2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -48.5dBm GSM Cell 3 Signal level: -48.5dBm Subtest 3: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -70.5dBm GSM Cell 3 Signal level: -70.5dBm Subtest 4: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -109.5dBm GSM Cell 3 Signal level: -109.5dBm Subtest 5: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -57.5dBm GSM Cell 3 Signal level: -54.5dBm Subtest 6: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -64.5dBm GSM Cell 3 Signal level: -59.5dBm Subtest 7: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -71.5dBm GSM Cell 3 Signal level: -64.5dBm Subtest 8: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -78.5dBm GSM Cell 3 Signal level: -69.5dBm Subtest 9: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -85.5dBm GSM Cell 3 Signal level: -74.5dBm Subtest 10: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -92.5dBm GSM Cell 3 Signal level: -79.5dBm Subtest 11: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -99.5dBm GSM Cell 3 Signal level: -84.5dBm Subtest 12: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -106.5dBm GSM Cell 3 Signal level: -89.5dBm |  |  | Subtest 1: E-UTRA Cell 1 0 dB 0 dB GSM Cell 2 -0.2 dB GSM Cell 3 0 dB Subtest 2: E-UTRA Cell 1 0 dB 0 dB GSM Cell 2 -0.2 dB GSM Cell 3 -1.5 dB Subtest 3: E-UTRA Cell 1 0 dB 0 dB GSM Cell 2 -0.2 dB GSM Cell 3 0 dB Subtest 4: E-UTRA Cell 1 0 dB 0 dB GSM Cell 2 +0.2 dB GSM Cell 3 0 dB Subtest 5: E-UTRA Cell 1 0 dB 0 dB GSM Cell 2 0.dB GSM Cell 3 0 dB Subtest 6: E-UTRA Cell 1 0 dB 0 dB GSM Cell 2 0dB GSM Cell 3 0 dB Subtest 7: E-UTRA Cell 1 0 dB 0 dB GSM Cell 2 0dB GSM Cell 3 0 dB Subtest 8: E-UTRA Cell 1 0 dB 0 dB GSM Cell 2 0dB GSM Cell 3 0 dB Subtest 9: E-UTRA Cell 1 0 dB 0 dB GSM Cell 2 0dB GSM Cell 3 0 dB Subtest 10: E-UTRA Cell 1 0 dB 0 dB GSM Cell 2 0dB GSM Cell 3 0 dB Subtest 11: E-UTRA Cell 1 0 dB 0 dB GSM Cell 2 0dB GSM Cell 3 0 dB Subtest 12: E-UTRA Cell 1 0 dB 0 dB GSM Cell 2 0dB GSM Cell 3 0 dB |  |  |  |  |  |  |  | Subtest 1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -38.7dBm GSM Cell 3 Signal level: -38.5dBm Subtest 2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -48.7dBm GSM Cell 3 Signal level: -50.0dBm Subtest 3: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -70.7dBm GSM Cell 3 Signal level: -70.5dBm Subtest 4: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -109.3dBm GSM Cell 3 Signal level: -109.5dBm Subtest 5: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -57.5dBm GSM Cell 3 Signal level: -54.5dBm Subtest 6: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -64.5dBm GSM Cell 3 Signal level: -59.5dBm Subtest 7: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -71.5dBm GSM Cell 3 Signal level: -64.5dBm Subtest 8: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -78.5dBm GSM Cell 3 Signal level: -69.5dBm Subtest 9: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -85.5dBm GSM Cell 3 Signal level: -74.5dBm Subtest 10: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -92.5dBm GSM Cell 3 Signal level: -79.5dBm Subtest 11: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -99.5dBm GSM Cell 3 Signal level: -84.5dBm Subtest 12: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -106.5dBm GSM Cell 3 Signal level: -89.5dBm |
| 9.6.2 GSM RSSI accuracy for E-UTRAN TDD |  |  | Same as 9.6.1 |  |  |  | Same as 9.6.1 |  |  |  |  | Same as 9.6.1 |  |  |
| 9.9.1.1 FDD Intra Frequency Serving Cell Absolute RSRP Accuracy |  |  | Noc: -122dBm to -118.5dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Reported RSRP values: ±6dB |  |  |  | 1.6dB 0dB Via mapping |  |  |  |  | Noc: -120.4dBm to -116.9dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB RSRP_9 to RSRP_24 RSRP_10 to RSRP_25 RSRP_10 to RSRP_25 RSRP_11 to RSRP_26 RSRP_12 to RSRP_27 RSRP_12 to RSRP_27 depending on operating band |  |  |
| 9.9.1.1_1 FDD Intra Frequency Serving Cell Absolute RSRP Accuracy (Rel 12 and forward) |  |  | Noc: -122dBm to -118.5dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Reported RSRP values: ±4.5dB for normal condition ±9dB for extreme condition |  |  |  | 1.6dB 0dB Via mapping |  |  |  |  | Noc: -120.4dBm to -116.9dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Normal condition: RSRP_10 to RSRP_22 RSRP_11 to RSRP_23 RSRP_11 to RSRP_23 RSRP_12 to RSRP_24 RSRP_12 to RSRP_24 RSRP_13 to RSRP_25 RSRP_14 to RSRP_26 depending on operating band Extreme condition: RSRP_6 to RSRP_27 RSRP_6 to RSRP_27 RSRP_7 to RSRP_28 RSRP_7 to RSRP_28 RSRP_8 to RSRP_29 RSRP_9 to RSRP_30 RSRP_9 to RSRP_30 depending on operating band |  |  |
| 9.9.1.2 FDD Intra Frequency Serving Cell Absolute RSRQ Accuracy |  |  | Noc: -122dBm to -118.5dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions |  |  | 1.6dB 0dB Via mapping |  |  |  |  |  |  |  | Noc: -120.4dBm to -116.9dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB RSRQ_00 to RSRQ_15 (NTC) RSRQ_00 to RSRQ_16 (ETC) |
| 9.9.2.1 TDD Intra Frequency Serving Cell Absolute RSRP Accuracy |  |  | Noc: -122dBm to -120dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Reported RSRP values: ±6dB |  |  | 1.6dB 0dB Via mapping |  |  |  |  |  |  |  | Noc: -120.4dBm to -118.4dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB RSRP_9 to RSRP_24 RSRP_10 to RSRP_25 RSRP_12 to RSRP_27 depending on operating band |
| 9.9.2.1_1 TDD Intra Frequency Serving Cell Absolute RSRP Accuracy (Rel 12 and forward) |  |  | Noc: -122dBm to -120dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Reported RSRP values: ±4.5dB for normal condition ±9dB for extreme condition |  |  | 1.6dB 0dB Via mapping |  |  |  |  |  |  |  | Noc: -120.4dBm to -118.4dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Normal condition: RSRP_10 to RSRP_22 RSRP_11 to RSRP_23 RSRP_12 to RSRP_24 depending on operating band Extreme condition: RSRP_6 to RSRP_27 RSRP_7 to RSRP_28 RSRP_8 to RSRP_29 depending on operating band |
| 9.9.2.2 TDD Intra Frequency Serving Cell Absolute RSRQ Accuracy |  |  | Noc: -122dBm to -120dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions |  |  | 1.6dB 0dB Via mapping |  |  |  |  |  |  |  | Noc: -120.4dBm to -118.4dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB RSRQ_00 to RSRQ_15 (NTC) RSRQ_00 to RSRQ_16 (ETC) |
| 9.11.1 FS3 average RSSI accuracy case (PCell using FDD) |  |  | Noc1: -106 dBm / 15kHz Noc2: -106 dBm / 15kHz outside RMTC Noc2: -83 dBm / 15kHz during RMTC Ês1 / Noc: 2.5dB Ês2 / Noc: 2.5dB outside RMTC Ês2 / Noc: -Infinity during RMTC Reported RSSI values: ±2.5dB for normal condition ±5.5dB for extreme condition |  |  | 0dB 0dB Via mapping |  |  |  |  |  |  |  | Noc1: -106 dBm / 15kHz Noc2: -106 dBm / 15kHz outside RMTC Noc2: -83 dBm / 15kHz during RMTC Ês1 / Noc: 2.5dB Ês2 / Noc: 2.5dB outside RMTC Ês2 / Noc: -Infinity during RMTC Reported RSSI values: RSSI_44 to RSSI_53 for normal condition RSSI_41 to RSSI_56 for extreme condition |
| 9.11.2 FS3 average RSSI accuracy case (PCell using TDD) |  |  | Same as 9.11.1 |  |  | Same as 9.11.1 |  |  |  |  |  |  |  | Same as 9.11.1 |
| 9.12.1 FS3 channel occupancy test (PCell using FDD) |  |  | Noc1: -106 dBm / 15kHz Noc2: -106 dBm / 15kHz outside RMTC Noc2: -96.28 dBm / 15kHz during RMTC and Channel Ocuppancy below threshold Noc2: -91.28 dBm / 15kHz during RMTC and Channel Ocuppancy above threshold Ês1 / Noc: 2.5dB Ês2 / Noc: 2.5dB outside RMTC Ês2 / Noc: -Infinity during RMTC |  |  | 0 dB 0 dB -1.89dB for normal conditions -4.89dB for extreme conditions 1.87dB for normal conditions 4.87dB for extreme conditions 0dB |  |  |  |  |  |  |  | Noc1: -106 dBm / 15kHz Noc2: -106 dBm / 15kHz outside RMTC Noc2: -98.17 dBm / 15kHz for NC and -101.17 dBm / 15 KHz for EC during RMTC and Channel Ocuppancy below threshold Noc2: -89.41 dBm / 15kHz for NC and -86.41 dBm / 15 KHz for EC during RMTC and Channel Ocuppancy above threshold Ês1 / Noc: 2.5dB Ês2 / Noc: 2.5dB outside RMTC Ês2 / Noc: -Infinity during RMTC |
| 9.12.2 FS3 channel occupancy test (PCell using TDD) |  |  | Same as 9.12.1 |  |  | Same as 9.12.1 |  |  |  |  |  |  |  | Same as 9.12.1 |
| 10.2 E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  |  | During T1: Formula: SNR + TT During T2: Formula: SNR - TT During T3: Formula: SNR+ TT |
| 10.4 E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication |  |  | During T1: E-UTRA Cell Noc: -98dBm /15kHz Ês / Noc: -infinity SyncRef UE Noc: -98dBm /15kHz Ês / Noc: 13.0dB During T2: E-UTRA Cell Noc: -98dBm /15kHz Ês / Noc: -3.0dB SyncRef UE Noc: -98dBm /15kHz Ês / Noc: 13.0dB |  |  | During T1: EUTRA Cell: 0dB 0dB SyncRef UE: 0dB 0dB During T2: EUTRA Cell 0dB 0.6 dB SyncRef UE : 0dB 0dB |  |  |  |  |  |  |  | During T1: E-UTRA Cell Noc: -98dBm /15kHz Ês / Noc: -infinity SyncRef UE Noc: -98dBm /15kHz Ês / Noc: 13.0dB During T2: E-UTRA Cell Noc: -98dBm /15kHz Ês / Noc: -2.4dB SyncRef UE Noc: -98dBm /15kHz Ês / Noc: 13.0 dB |
| 11.1 V2V UE Transmission Timing Accuracy Test |  |  | Uplink timing: ±12Ts GNSS Signal Power: GPS: -128.5dBm Galileo: -127dBm GLONASS: -131dBm BDS: -133dBm |  |  | ±3Ts 0dB 0dB 0dB 0dB |  |  |  |  |  |  |  | Uplink timing: ±15Ts GNSS Signal Power: GPS: -128.5dBm Galileo: -127dBm GLONASS: -131dBm BDS: -133dBm |
| 11.2 Interruptions due to V2V sidelink communication |  |  | Noc: -98dBm /15kHz Ês / Noc: 16dB GNSS Signal Power: GPS: -128.5dBm Galileo: -127dBm GLONASS: -131dBm BDS: -133dBm |  |  | 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  |  |  |  | Noc: -98dBm /15kHz Ês / Noc: 16dB GNSS Signal Power: GPS: -128.5dBm Galileo: -127dBm GLONASS: -131dBm BDS: -133dBm |
| 12.1.1 V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference |  |  | Uplink timing: ±12Ts GNSS Signal Power: GPS: -128.5dBm Galileo: -127dBm GLONASS: -131dBm BDS: -133dBm |  |  | ±3Ts 0dB 0dB 0dB 0dB |  |  |  |  |  |  |  | Uplink timing: ±15Ts GNSS Signal Power: GPS: -128.5dBm Galileo: -127dBm GLONASS: -131dBm BDS: -133dBm |
| 12.1.2 V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference |  |  | Uplink timing: ±12Ts |  |  | ±3Ts |  |  |  |  |  |  |  | Uplink timing: ±15Ts |
| 12.2.1 Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference |  |  | During T1: Noc: -110dBm/15kHz Ês / Noc: +4.5dB During T2: Noc: -110dBm/15kHz Ês / Noc: -4.5dB During T3: Noc: -110dBm/15kHz Ês / Noc: +4.5dB |  |  | During T1: 0dB 1.1dB During T2: 0dB -1.1dB During T3: 0dB 1.1dB |  |  |  |  |  |  |  | During T1: Noc: -110dBm/15kHz Ês / Noc: +5.6dB During T2: Noc: -110dBm/15kHz Ês / Noc: -5.6dB During T3: Noc: -110dBm/15kHz Ês / Noc: +5.6dB |
| 12.3.1 V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority |  |  | During T1: Noc: -95dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: -infinity During T2: Noc: -95dBm/15kHz Ês1 / Noc: 0dB Ês2 / Noc: -infinity During T3: Noc: -95dBm/15kHz Ês1 / Noc: 0dB Ês2 / Noc: 3dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0.3dB 0dB During T3: 0dB 0.3dB 0.5dB |  |  |  |  |  |  |  | During T1: Noc: -95dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: -infinity During T2: Noc: -95dBm/15kHz Ês1 / Noc: 0.3dB Ês2 / Noc: -infinity During T3: Noc: -95dBm/15kHz Ês1 / Noc: 0.3dB Ês2 / Noc: 3.5dB |
| 12.3.2 V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority |  |  | During T1: Noc: -95dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: -infinity During T2: Noc: -95dBm/15kHz Ês1 / Noc: 0dB Ês2 / Noc: -infinity During T3: Noc: -95dBm/15kHz Ês1 / Noc: 0dB Ês2 / Noc: 3dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0.3dB 0dB During T3: 0dB 0.3dB 0.5dB |  |  |  |  |  |  |  | During T1: Noc: -95dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: -infinity During T2: Noc: -95dBm/15kHz Ês1 / Noc: 0.3dB Ês2 / Noc: -infinity During T3: Noc: -95dBm/15kHz Ês1 / Noc: 0.3dB Ês2 / Noc: 3.5dB |
| 12.2.2 Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference |  |  | During T1: Noc: -101dBm/15kHz Ês / Noc: +5.5dB During T2: Noc: -101dBm/15kHz Ês / Noc: -3.5dB During T3: Noc: -101dBm/15kHz Ês / Noc: +5.5dB |  |  | During T1: -1.6dB 3.2dB During T2: -1.6dB 0dB During T3: -1.6dB 3.2dB |  |  |  |  |  |  |  | During T1: Noc: -102.6dBm/15kHz Ês / Noc: +8.7dB During T2: Noc: -102.6dBm/15kHz Ês / Noc: -3.5dB During T3: Noc: -102.6dBm/15kHz Ês / Noc: +8.7dB |
| 12.4 Congestion Control Measurement Test for V2X UE |  |  | During T1: Noc: -106dBm/15kHz PUCCH Ês / Noc: +7.19dB During T2: Noc: -106dBm/15kHz PUCCH Ês / Noc: +13.19dB |  |  | During T1: 0dB -2.3dB During T2: 0dB 1.7dB |  |  |  |  |  |  |  | During T1: Noc: -106dBm/15kHz PUCCH Ês / Noc: +4.89dB During T2: Noc: -106dBm/15kHz PUCCH Ês / Noc: +14.89dB |
| 12.5 Interruptions due to V2X Sidelink Communication |  |  | During T1, T2 and T3: Noc: -98dBm/15kHz Ês / Noc: 16dB |  |  | During T1: 0dB 0dB |  |  |  |  |  |  |  | During T1, T2 and T3: Noc: -98dBm/15kHz Ês / Noc: 16dB |
| 12.6.1 V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements |  |  | During T1: Noc: -103dBm/15kHz PSSCH Ês / Noc: 2dB During T2: Noc: -113dBm/15kHz PSSCH Ês / Noc: 2dB |  |  | During T1: 0dB 4.1dB During T2: 0dB -4.1dB |  |  |  |  |  |  |  | During T1: Noc: -103dBm/15kHz PSSCH Ês / Noc: 6.1dB During T2: Noc: -113dBm/15kHz PSSCH Ês / Noc: -2.1dB |
| 12.6.2 V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements |  |  | During T1: Noc: -105dBm/15kHz PSSCH Ês / Noc: 2dB |  |  | During T1: 0dB 0dB |  |  |  |  |  |  |  | During T1: Noc: -105dBm/15kHz PSSCH Ês / Noc: 2dB |
| 13.1.1.1 HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage |  |  | During T1: Noc: -98dBm/15KHz Êsn1 / Noc: 17.00dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15KHz Êsn1 / Noc: 13.00dB Êsn2 / Noc: 17.00dB During T3: Noc: -98dBm/15KHz Êsn1 / Noc: 17.00dB Êsn2 / Noc: 13.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.45dB During T3: 0dB +0.45dB 0dB |  |  |  |  |  |  |  | During T1: Noc: -98dBm/15KHz Êsn1 / Noc: 17.00dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15KHz Êsn1 / Noc 13.00dB Êsn2 / Noc: 17.45dB During T3: Noc: -98dBm/15KHz Êsn1 / Noc: 17.45dB Êsn2 / Noc: 13.00dB |
| 13.1.1.1_1 HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage for GSO and NGSO |  |  | Same as 13.1.1.1 |  |  | Same as 13.1.1.1 |  |  |  |  |  |  |  | Same as 13.1.1.1 |
| 13.1.1.2 HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with serving cell RRM measurement relaxation |  |  | During T1: Noc: -98dBm/15kHz Êsn1 / Noc: 17dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Êsn1 / Noc: 7dB Êsn2 / Noc: 11dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.45dB |  |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Êsn1 / Noc: 17dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Êsn1 / Noc: 7dB Êsn2 / Noc: 11.45dB |
| 13.1.1.2_1 HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with serving cell RRM measurement relaxation for GSO and NGSO |  |  | Same as 13.1.1.2 |  |  | Same as 13.1.1.2 |  |  |  |  |  |  |  | Same as 13.1.1.2 |
| 13.1.1.3 HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX |  |  | Same as 13.1.1.1 |  |  | Same as 13.1.1.1 |  |  |  |  |  |  |  | Same as 13.1.1.1 |
| 13.1.1.3_1 FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX for GSO and NGSO |  |  | Same as 13.1.1.1 |  |  | Same as 13.1.1.1 |  |  |  |  |  |  |  | Same as 13.1.1.1 |
| 13.1.1.4 HD – FDD Inter frequency case for UE Category NB1 Standalone mode in normal coverage |  |  | During T1: Noc: -98dBm/15KHz Êsn1 / Noc: 17.00dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15KHz Êsn1 / Noc: 13.00dB Êsn2 / Noc: 17.00dB During T3: Noc: -98dBm/15KHz Êsn1 / Noc: 17.00dB Êsn2 / Noc: 13.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +1.5dB During T3: 0dB +1.5dB 0dB |  |  |  |  |  |  |  | During T1: Noc: -98dBm/15KHz Êsn1 / Noc: 17.00dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15KHz Êsn1 / Noc 13.00dB Êsn2 / Noc: 18.5dB During T3: Noc: -98dBm/15KHz Êsn1 / Noc: 18.5dB Êsn2 / Noc: 13.00dB |
| 13.1.1.5 HD – FDD Intra frequency case for UE Category NB1 Standalone mode in enhanced coverage, location-based cell reselection for NGSO |  |  | During T0: Noc: -98dBm/15KHz Êsn1 / Noc: 10.00dB Êsn2 / Noc: -infinity During T1: Noc: -98dBm/15KHz Êsn1 / Noc: -9.00dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15KHz Êsn1 / Noc: -9.00dB Êsn2 / Noc: -0.7dB |  |  | During T0: 0dB 0dB 0dB During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.5dB |  |  |  |  |  |  |  | During T0: Noc: -98dBm/15KHz Êsn1 / Noc: 10.00dB Êsn2 / Noc: -infinity During T1: Noc: -98dBm/15KHz Êsn1 / Noc: -9.00dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15KHz Êsn1 / Noc: -9.00dB Êsn2 / Noc: -0.2dB |
| 13.1.1.6 FDD Inter frequency case for UE Category NB1 Standalone mode in enhanced coverage, time-based cell reselection for NGSO |  |  | During T1: Noc: -98dBm/15KHz Êsn1 / Noc: 10.00dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15KHz Êsn1 / Noc: -12.00dB Êsn2 / Noc: -infinity During T3: Noc: -98dBm/15KHz Êsn1 / Noc: -12.00dB Êsn2 / Noc: -2.7dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB +0.5dB |  |  |  |  |  |  |  | During T1: Noc: -98dBm/15KHz Êsn1 / Noc: 10.00dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15KHz Êsn1 / Noc -12.00dB Êsn2 / Noc: -infinity During T3: Noc: -98dBm/15KHz Êsn1 / Noc: -12dB Êsn2 / Noc: -2.2dB |
| 13.3.1.1 HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage |  |  | During T1: Noc: -98dBm/15kHz Êsn1 / Noc: +7.0dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Êsn1 / Noc: -infinity Êsn2 / Noc: +4.0dB During T3: Noc: -98dBm /15kHz Êsn1 / Noc: -infinity Êsn2 / Noc: +4.0dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB |  |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Êsn1 / Noc: +7.0dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Êsn1 / Noc: -infinity Êsn2 / Noc: +4.0dB During T3: Noc: -98dBm /15kHz Êsn1 / Noc: -infinity Êsn2 / Noc: +4.0dB |
| 13.3.1.1_1 HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage for GSO and NGSO |  |  | Same as 13.3.1.1 |  |  | Same as 13.3.1.1 |  |  |  |  |  |  |  | Same as 13.3.1.1 |
| 13.3.1.2 HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage |  |  | During T1: Noc: -98dBm/15kHz Êsn1 / Noc: +7.0dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Êsn1 / Noc: -infinity Êsn2 / Noc: -12.6dB During T3: Noc: -98dBm /15kHz Êsn1 / Noc: -infinity Êsn2 / Noc: -12.6dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB |  |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Êsn1 / Noc: +7.0dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Êsn1 / Noc: -infinity Êsn2 / Noc: -12.6dB During T3: Noc: -98dBm /15kHz Êsn1 / Noc: -infinity Êsn2 / Noc: -12.6dB |
| 13.3.1.2_1 HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage for GSO and NGSO |  |  | Same as 13.3.1.2 |  |  | Same as 13.3.1.2 |  |  |  |  |  |  |  | Same as 13.3.1.2 |
| 13.3.2.1 Contention Based Random Access Test for UE category NB1 UEs Standalone mode in normal coverage |  |  | Absolute uplink power: Normal conditions ±9dB Extreme conditions ±12dB Relative uplink power step: Normal conditions ±2dB Extreme conditions ±4dB Uplink timing Te: ±97Ts rsrp-ThresholdsPrach {40, 55} Ês / Noc: 3dB |  |  | 1.4dB 1.4dB 0.7dB 0.7dB 3Ts {0, -1} for normal {0, -4} for extreme 0.1dB |  |  |  |  |  |  |  | Absolute uplink power: Normal conditions ±10.4dB Extreme conditions ±13.4dB Relative uplink power step: Normal conditions ±2.7dB Extreme conditions ±4.7dB Uplink timing Te: ±100Ts rsrp-ThresholdsPrach {40, 54} for normal conditions and {40, 51} for extreme conditions Ês / Noc: 3.1dB |
| 13.3.2.2 Contention Based Random Access Test for UE category NB1 UEs Standalone mode in Enhanced Coverage |  |  | Absolute uplink power: Normal conditions ±13.3dB Extreme conditions ±16.3dB Relative uplink power step: Normal conditions ±2dB Extreme conditions ±4dB Uplink timing Te: ±97Ts rsrp-ThresholdsPrach {35, 56} |  |  | 1.0dB 1.0dB 0.7dB 0.7dB 3Ts {-1, 1} for normal {-4, 4} for extreme |  |  |  |  |  |  |  | Absolute uplink power: Normal conditions ±14.3dB Extreme conditions ±17.3dB Relative uplink power step: Normal conditions ±2.7dB Extreme conditions ±4.7dB Uplink timing Te: ±100Ts rsrp-ThresholdsPrach {34, 57} for normal conditions and {31, 60} for extreme conditions |
| 13.3.2.3 Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs Standalone mode in Enhanced Coverage |  |  | Same as 13.3.2.2 |  |  | Same as 13.3.2.2 |  |  |  |  |  |  |  | Same as 13.3.2.2 |
| 13.4.1.1 E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under normal coverage for Satellite Access |  |  | Uplink timing: ±( 97*Ts- TGNSS_margin) Max step size: 58.33*Ts Min adjustment rate: 7*Ts Max adjustment rate: 58.33*Ts Ês / Noc: +4.0dB TGNSS_margin is specified in TS 36.133 A.13.4.1.1.2 |  |  | ±3Ts +1.4Ts for GSO +0.7Ts for NGSO -3.6Ts +1.1Ts +0.3dB |  |  |  |  |  |  |  | Uplink timing: ±(100 *Ts- TGNSS_margin) Max step size: 59.73*Ts for GSO 59.03*Ts for NGSO Min adjustment rate: 3.4 *Ts Max adjustment rate: 59.43*Ts Ês / Noc: +4.3dB |
| 13.4.1.2 E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage for Satellite Access |  |  | Uplink timing: ±(97*Ts- TGNSS_margin) Uplink timing: 0Ts (during ongoing repetition) Ês / Noc: -11.0dB TGNSS_margin is specified in TS 36.133 A.13.4.1.2.2 |  |  | ±3Ts ±3Ts +0.3dB |  |  |  |  |  |  |  | Uplink timing: ±(100 *Ts- TGNSS_margin) Uplink timing: ±3Ts (during ongoing repetition) Ês / Noc: -10.7dB |
| 13.4.1.3 E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage with segment transmission in NGSO for Satellite Access |  |  | Same as 13.4.1.2 |  |  | Same as 13.4.1.2 |  |  |  |  |  |  |  | Same as 13.4.1.2 |
| 13.4.2.1 HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage for Satellite Access |  |  | Timing Advance Adjustment: ±13.33Ts |  |  | ±0.5Ts |  |  |  |  |  |  |  | Timing Advance Adjustment: ±13.83Ts |
| 13.4.2.2 HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhance Coverage for Satellite Access |  |  | Same as 13.4.2.1 |  |  | Same as 13.4.2.1 |  |  |  |  |  |  |  | Same as 13.4.2.1 |
| 13.4.3.1 HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in normal coverage |  |  | During T1: SNR1: -3.1dB During T2: SNR2: -9.1dB During T3: SNR3: -14.1dB During T4: SNR1: -3.1dB |  |  | During T1: +0.3dB During T2: +0.3dB During T3: -0.3dB During T4: +0.3dB |  |  |  |  |  |  |  | During T1: SNR1: -2.8dB During T2: SNR2: -8.8dB During T2: SNR3: -14.4dB During T3: SNR1: -2.8dB |
| 13.4.3.2 HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in enhanced coverage |  |  | During T1: SNR1: -6.3dB During T2: SNR2: -11.4dB During T3: SNR3: -17.4dB During T4: SNR1: -6.3dB |  |  | During T1: +0.4dB During T2: +0.3dB During T3: -0.3dB During T4: +0.4dB |  |  |  |  |  |  |  | During T1: SNR1: -5.9dB During T2: SNR2: -11.1dB During T2: SNR3: -17.7dB During T3: SNR1: -5.9dB |
| 13.4.3.3 HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 Standalone mode in Enhanced Coverage |  |  | During T1: SNR1: -6.3dB T2 duration: 2.12s During T2: SNR2: -17.4dB During T3: SNR1: -6.3dB |  |  | During T1: +0.4dB T2 duration: -0.16s During T2: -0.3dB During T3: +0.4dB |  |  |  |  |  |  |  | During T1: SNR1: -5.9dB T2 duration: 1.96s During T2: SNR2: -17.7dB During T3: SNR1: -5.9dB |
| 13.4.3.4 HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 Standalone mode in Normal Coverage |  |  | During T1: SNR1: -3.1dB T2 duration: 2.12s During T2: SNR2: -14.1dB During T3: SNR1: -3.1dB |  |  | During T1: +0.3dB T2 duration: -0.16s During T2: -0.3dB During T3: +0.3dB |  |  |  |  |  |  |  | During T1: SNR1: -2.8dB T2 duration: 1.96s During T2: SNR2: -14.4dB During T3: SNR1: -2.8dB |
| 13.4.3.5 HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage |  |  | Same as 13.4.3.4 |  |  | Same as 13.4.3.4 |  |  |  |  |  |  |  | Same as 13.4.3.4 |
| 13.4.3.6 HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 Standalone mode in Enhanced Coverage |  |  | Same as 13.4.3.3 |  |  | Same as 13.4.3.3 |  |  |  |  |  |  |  | Same as 13.4.3.3 |
| 13.4.3.7 HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage |  |  | Same as 13.4.3.1 |  |  | Same as 13.4.3.1 |  |  |  |  |  |  |  | Same as 13.4.3.1 |
| 13.4.3.8 HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Enhanced Coverage |  |  | Same as 13.4.3.2 |  |  | Same as 13.4.3.2 |  |  |  |  |  |  |  | Same as 13.4.3.2 |
| 13.5.1 HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access |  |  | During T1: Noc: -98dBm/15kHz Êsn1 / Noc: +9.0dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Êsn1 / Noc: -3.0dB Êsn2 / Noc: -infinity During T3: Noc: -98dBm /15kHz Êsn1 / Noc: -8.5dB Êsn2 / Noc: +2.2dB During T4: Noc: -98dBm /15kHz Êsn1 / Noc: -infinity Êsn2 / Noc: +4.0dB During T5: Noc: -98dBm /15kHz Êsn1 / Noc: -infinity Êsn2 / Noc: +4.0dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB +3.0dB 0dB During T4: 0dB 0dB 0dB During T5: 0dB 0dB 0dB |  |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Êsn1 / Noc: +9.0dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Êsn1 / Noc: -3.0dB Êsn2 / Noc: -infinity During T3: Noc: -98dBm /15kHz Êsn1 / Noc: -5.5dB Êsn2 / Noc: +2.2dB During T4: Noc: -98dBm /15kHz Êsn1 / Noc: -infinity Êsn2 / Noc: +4.0dB During T5: Noc: -98dBm /15kHz Êsn1 / Noc: -infinity Êsn2 / Noc: +4.0dB |
| 13.5.2 HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access |  |  | During T1: Noc: -98dBm/15kHz Êsn1 / Noc: +9.0dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Êsn1 / Noc: -3.0dB Êsn2 / Noc: -infinity During T3: Noc: -98dBm /15kHz Êsn1 / Noc: -3.0dB Êsn2 / Noc: +4.0dB During T4: Noc: -98dBm /15kHz Êsn1 / Noc: -infinity Êsn2 / Noc: +4.0dB During T5: Noc: -98dBm /15kHz Êsn1 / Noc: -infinity Êsn2 / Noc: +4.0dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB During T4: 0dB 0dB 0dB During T5: 0dB 0dB 0dB |  |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Êsn1 / Noc: +9.0dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Êsn1 / Noc: -3.0dB Êsn2 / Noc: -infinity During T3: Noc: -98dBm /15kHz Êsn1 / Noc: -3.0dB Êsn2 / Noc: +4.0dB During T4: Noc: -98dBm /15kHz Êsn1 / Noc: -infinity Êsn2 / Noc: +4.0dB During T5: Noc: -98dBm /15kHz Êsn1 / Noc: -infinity Êsn2 / Noc: +4.0dB |
| 13.6.2.1 HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  |  | Formula: SNR + TT |
| 13.6.2.2 HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage |  |  | SNRs as specified |  |  | 0dB |  |  |  |  |  |  |  | SNRs as specified |
| 13.6.2.3 HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage |  |  | SNRs as specified |  |  | Same as 13.6.2.1 |  |  |  |  |  |  |  | Same as 13.6.2.1 |
| 13.6.2.4 HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage |  |  | SNRs as specified |  |  | Same as 13.6.2.2 |  |  |  |  |  |  |  | Same as 13.6.2.2 |
| 13.6.2.5 HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under normal coverage |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  |  | Formula: SNR + TT |
| 13.6.2.6 HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under enhanced coverage |  |  | SNRs as specified |  |  | Same as 13.6.2.5 |  |  |  |  |  |  |  | Same as 13.6.2.5 |

Annex G (normative):
Statistical Testing

# G.1	General

FSS

# G.2	Statistical testing of delay and UE measurement performance in RRM tests

## G.2.1	General

The RRM tests are either of deterministic or of statistical nature. The pass fail limits in tests of statistical nature are expressed as a limit ( e.g. delay limit) and a success ratio applicable for the limit. The success ratio is 90% uniform (the complement is the error ratio ER = 10%).

## G.2.2	Design of the test

The test is defined by the following design principles (see 3GPP TS 36.521-1 clause G.X, Theory):

1)	The early decision concept is applied.

2)	A second limit is introduced: bad DUT factor M>1

To decide the test pass:

Supplier risk is applied based on the bad DUT quality

To decide the test fails

Customer risk is applied based on the specified DUT quality

The test is defined by the following parameters:

1)	Limit ER = 0.1 (success ratio = 90%)

2)	Bad DUT factor M=1.5 (selectivity)

3)	Confidence level CL = 95% (for specified DUT and bad DUT-quality)

## G.2.3	Numerical definition of the pass fail limits

Table G.2.3-1: pass fail limits

| ne | nsp | nsf | ne | nsp | nsf | ne | nsp | nsf | ne | nsp | nsf |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 33 | NA | 43 | 408 | 283 | 86 | 737 | 644 | 129 | 1056 | 1021 |
| 1 | 46 | NA | 44 | 416 | 291 | 87 | 745 | 653 | 130 | 1064 | 1030 |
| 2 | 58 | 2 | 45 | 424 | 299 | 88 | 752 | 661 | 131 | 1071 | 1039 |
| 3 | 69 | 5 | 46 | 432 | 307 | 89 | 760 | 670 | 132 | 1078 | 1048 |
| 4 | 79 | 8 | 47 | 440 | 315 | 90 | 767 | 679 | 133 | 1086 | 1057 |
| 5 | 89 | 12 | 48 | 447 | 324 | 91 | 775 | 687 | 134 | 1093 | 1066 |
| 6 | 99 | 17 | 49 | 455 | 332 | 92 | 782 | 696 | 135 | 1100 | 1074 |
| 7 | 109 | 22 | 50 | 463 | 340 | 93 | 790 | 705 | 136 | 1108 | 1083 |
| 8 | 118 | 27 | 51 | 471 | 348 | 94 | 797 | 713 | 137 | 1115 | 1092 |
| 9 | 127 | 33 | 52 | 478 | 356 | 95 | 804 | 722 | 138 | 1122 | 1101 |
| 10 | 136 | 39 | 53 | 486 | 365 | 96 | 812 | 731 | 139 | 1130 | 1110 |
| 11 | 145 | 45 | 54 | 494 | 373 | 97 | 819 | 739 | 140 | 1137 | 1119 |
| 12 | 154 | 51 | 55 | 502 | 381 | 98 | 827 | 748 | 141 | 1144 | 1128 |
| 13 | 163 | 58 | 56 | 509 | 389 | 99 | 834 | 757 | 142 | 1152 | 1137 |
| 14 | 172 | 64 | 57 | 517 | 398 | 100 | 842 | 766 | 143 | 1159 | 1147 |
| 15 | 180 | 71 | 58 | 525 | 406 | 101 | 849 | 774 | 144 | 1166 | 1155 |
| 16 | 189 | 78 | 59 | 532 | 414 | 102 | 857 | 783 | 145 | 1174 | 1164 |
| 17 | 197 | 85 | 60 | 540 | 423 | 103 | 864 | 792 | 146 | 1181 | 1173 |
| 18 | 206 | 92 | 61 | 548 | 431 | 104 | 871 | 801 | 147 | NA | 1182 |
| 19 | 214 | 99 | 62 | 555 | 440 | 105 | 879 | 809 | 148 |  |  |
| 20 | 223 | 106 | 63 | 563 | 448 | 106 | 886 | 818 | 149 |  |  |
| 21 | 231 | 113 | 64 | 571 | 456 | 107 | 894 | 827 | 150 |  |  |
| 22 | 239 | 120 | 65 | 578 | 465 | 108 | 901 | 836 | 151 |  |  |
| 23 | 248 | 128 | 66 | 586 | 473 | 109 | 909 | 844 | 152 |  |  |
| 24 | 256 | 135 | 67 | 594 | 482 | 110 | 916 | 853 | 153 |  |  |
| 25 | 264 | 142 | 68 | 601 | 490 | 111 | 923 | 862 | 154 |  |  |
| 26 | 272 | 150 | 69 | 609 | 499 | 112 | 931 | 871 | 155 |  |  |
| 27 | 281 | 157 | 70 | 616 | 507 | 113 | 938 | 880 | 156 |  |  |
| 28 | 289 | 165 | 71 | 624 | 516 | 114 | 946 | 888 | 157 |  |  |
| 29 | 297 | 173 | 72 | 632 | 524 | 115 | 953 | 897 | 158 |  |  |
| 30 | 305 | 180 | 73 | 639 | 533 | 116 | 960 | 906 | 159 |  |  |
| 31 | 313 | 188 | 74 | 647 | 541 | 117 | 968 | 915 | 160 |  |  |
| 32 | 321 | 196 | 75 | 654 | 550 | 118 | 975 | 924 | 161 |  |  |
| 33 | 329 | 204 | 76 | 662 | 558 | 119 | 983 | 933 | 162 |  |  |
| 34 | 337 | 211 | 77 | 669 | 567 | 120 | 990 | 941 | 163 |  |  |
| 35 | 345 | 219 | 78 | 677 | 575 | 121 | 997 | 950 | 164 |  |  |
| 36 | 353 | 227 | 79 | 684 | 584 | 122 | 1005 | 959 | 165 |  |  |
| 37 | 361 | 235 | 80 | 692 | 592 | 123 | 1012 | 968 | 166 |  |  |
| 38 | 369 | 243 | 81 | 700 | 601 | 124 | 1019 | 977 | 167 |  |  |
| 39 | 377 | 251 | 82 | 707 | 610 | 125 | 1027 | 986 | 168 |  |  |
| 40 | 385 | 259 | 83 | 715 | 618 | 126 | 1034 | 994 | 169 |  |  |
| 41 | 393 | 267 | 84 | 722 | 627 | 127 | 1042 | 1003 |  |  |  |
| 42 | 400 | 275 | 85 | 730 | 635 | 128 | 1049 | 1012 |  |  |  |

The first column is the number of errors (ne = number of exceeded delays or number of wrong reports)

The second column is the number of samples for the pass limit (nsp , ns=Number of samples= number of successes + number of exceedings or number of reports)

The third column is the number of samples for the fail limit (nsf)

## G.2.4	Pass fail decision rules

The pass fail decision rules apply for a single test, comprising one component in the test vector. The over all Pass /Fail conditions are defined in clause G.2.6

Having observed 0 errors, pass the test at 33+ samples, otherwise continue

Having observed 1 error, pass the test at 46+ samples, otherwise continue

Having observed 2 errors, pass the test at 58+ samples, fail the test at 2 samples, otherwise continue

Having observed 146 errors, pass the test at 1181+ samples, fail the test at 1173- samples, otherwise continue

Having observed 147 errors, fail the test at 1182- samples,

Where x+ means: x or more, x- means x or less

NOTE:	An ideal DUT passes after 33 samples. The maximum test time is 1181 samples.

## G.2.5	Void

## G.2.6	Test conditions for delay tests and UE measurement performance

Table G.2.6-1: test conditions

| Test | Statistical independence | Number of components in the test vector, as specified in the test requirements and initial conditions of the applicable test | - | Over all Pass/Fail condition |
| --- | --- | --- | --- | --- |
| All tests in clauses 4, 5, 6.1, 7.2, 7.3 and 8 are delay tests of statistical nature while 6.2 and 7.1 are not applicable, since deterministic. | Test procedure in all statistical tests ensures independency | 1 per operating band (if tested, see 3A.3.3) |  | Full set of environmental conditions (5) per operating band |
| All tests in clause 9 are UE level reports of statistical nature | Independency is assumed, although Layer 1 filtering is applied to the reported results | Full set of environmental conditions (5) per operating band |  | Full set of environmental conditions (5) per operating band |

# G.X	Theory to derive the numbers in Table G.2.3-1 (informative)

3GPP TS 36.521-1 Annex G.X applies

Annex H (normative):
Default Message Contents

This annex contains the default values of common messages specific to RRM, other than those described in 3GPP TS 36.508 [7]. The message contents shall apply to test cases accordingly and unless indicated otherwise in specific test cases, shall be transmitted and checked by the system simulator. The default message contents can be defined for FDD Mode, or TDD Mode or both FDD/TDD Modes. All the messages are listed in alphabetical order based on conformance tests.

NOTE:	For example, test case 8.1.1 has an exception for RRCConnectionReconfiguration message and therefore uses message contents according to TS 36.508 [7] with the exception of the RRCConnectionReconfiguration message specified in Annex H.

# H.1	Common contents of system information messages exceptions

This clause contains the default values of common system information messages, other than those described in 3GPP TS 36.508 [7].

# H.2	Common contents of system information blocks exceptions

This clause contains the default values of common system information blocks, other than those described in 3GPP TS 36.508 [7].

## H.2.1	System information blocks message contents exceptions for E-UTRAN intra frequency cell re-selection

SystemInformationBlockType3: (FDD/TDD) for E-UTRAN intra-frequency cell re-selection

Table H.2.1-1: SystemInformationBlockType3: E-UTRAN intra frequency cell re-selection

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-2 SystemInformationBlockType3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| cellReselectionInfoCommon SEQUENCE { |  |  |  |
| q-Hyst | dB0 (0 dB) | 0 is actual value in dB (0 * 2 dB) |  |
| } |  |  |  |

SystemInformationBlockType4: (FDD/TDD) for E-UTRAN intra-frequency cell re-selection

For Cell 2

Table H.2.1-2: SystemInformationBlockType4: E-UTRAN intra frequency cell re-selection

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-3 SystemInformationBlockType4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| intraFreqNeighCellList SEQUENCE (SIZE (1..maxCellIntra)) OF SEQUENCE { |  |  |  |
| IntraFreqNeighCellInfo ::= SEQUENCE { |  |  |  |
| physCellId | 0 (Cell 1 Id) | INTEGER (0..503) |  |
| q-OffsetCell | dB0 (0 dB) | 0 is actual value in dB (0 * 2 dB) |  |
| } |  |  |  |
| } |  |  |  |

## H.2.2	System information blocks message contents exceptions for E-UTRAN inter frequency cell re-selection

SystemInformationBlockType3: (FDD/TDD) for E-UTRAN inter-frequency cell re-selection

Table H.2.2-1: SystemInformationBlockType3: E-UTRAN inter frequency cell re-selection

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-2 SystemInformationBlockType3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearch | 25 (50 dB) | 50 is actual value in dB (25 * 2 dB); for Cell 1 | Cell 1 |
|  | Not Present |  | Cell 2 |
| threshServingLow | 22 (44 dB) | 44 is actual value in dB (22 * 2 dB) |  |
| cellReselectionPriority | 4 |  | Cell 1 |
|  | 5 |  | Cell 2 |
| } |  |  |  |

SystemInformationBlockType5: (FDD/TDD) for E-UTRAN inter-frequency cell re-selection case

For Cell 1

Table H.2.2-2: SystemInformationBlockType5: E-UTRAN inter frequency cell re-selection

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-4 SystemInformationBlockType5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| q-Rxlevmin | -70 (-140 dBm) | -140 is actual value in dBm (-70 * 2 dBm) |  |
| threshX-High | 24 (48 dB) | 48 is actual value in dB (24 * 2 dB) |  |
| threshX-Low | 25 (50 dB) | 50 is actual value in dB (25 * 2 dB) |  |
| cellReselectionPriority[n] | 5 for cell 1 |  |  |
| } |  |  |  |

For Cell 2

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-4 SystemInformationBlockType5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| q-Rxlevmin | -70 (-140 dBm) | -140 is actual value in dBm (-70 * 2 dBm) |  |
| threshX-High | 24 (48 dB) | 48 is actual value in dB (24 * 2 dB) |  |
| threshX-Low | 25 (50 dB) | 50 is actual value in dB (25 * 2 dB) |  |
| cellReselectionPriority[n] | 4 for cell 2 |  |  |
| } |  |  |  |
| interFreqNeighCellList[n] SEQUENCE (SIZE (1..maxCellInter)) OF SEQUENCE { |  |  |  |
| physCellId | 0 (Cell 1 Id) | INTEGER (0..503) |  |
| q-OffsetCell | dB0 (0 dB) | 0 is actual value in dB (0 * 2 dB) |  |
| } |  |  |  |

## H.2.3	System information blocks message contents exceptions for E-UTRAN inter-RAT cell re-selection

SystemInformationBlockType3: for inter-RAT EUTRAN FDD - UTRA FDD is of higher priority cell re-selection

Table H.2.3-1: SystemInformationBlockType3: Inter-RAT E-UTRAN FDD - UTRA FDD is of higher priority cell re-selection

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-2 SystemInformationBlockType3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearch | 25 (50 dB) | 50 is actual value in dB (25 * 2 dB); for Cell 1 (E-UTRA) |  |
| } |  |  |  |

SystemInformationBlockType6: for inter-RAT EUTRAN FDD - UTRA FDD is of higher priority cell re-selection

Table H.2.3-2: SystemInformationBlockType6: Inter-RAT E-UTRAN FDD - UTRA FDD is of higher priority cell re-selection

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-5 SystemInformationBlockType6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CarrierFreqListUTRA-FDD ::= SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| threshX-High | 20 (40 dB) | 40 is actual value in dB (20 * 2 dB) |  |
| q-RxLevMin | -58 (-115 dBm) | -115 is actual value in dBm (-58 * 2 + 1 dBm) |  |
| p-MaxUTRA | 21 (21 dBm) | Default value in 3GPP TS 36.508 |  |
| q-QualMin | -20 (-20 dB) |  |  |
| cellReselectionPriority[n] | 5 | UTRA is of higher priority than E-UTRAN |  |
| } |  |  |  |

SystemInformationBlockType3: for inter-RAT EUTRAN TDD - UTRA TDD is of higher priority cell re-selection

Table H.2.3-3: SystemInformationBlockType3: Inter-RAT E-UTRAN TDD - UTRA TDD is of higher priority cell re-selection

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-2 SystemInformationBlockType3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearch | 23 (46 dB) | 46 is actual value in dB (23 * 2 dB); for Cell 1 (E-UTRA) |  |
| } |  |  |  |

SystemInformationBlockType6: for inter-RAT EUTRAN TDD - UTRA TDD is of higher priority cell re-selection

Table H.2.3-4: SystemInformationBlockType6: Inter-RAT E-UTRAN TDD - UTRA TDD is of higher priority cell re-selection

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-5 SystemInformationBlockType6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CarrierFreqListUTRA-TDD ::= SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { |  |  | UTRA-TDD |
| threshX-High | 12 (24 dB) | 24 is actual value in dB (12 * 2 dB) |  |
| q-RxLevMin | -52 (-103 dBm) | -103 is actual value in dBm (-52 * 2 + 1 dBm) |  |
| p-MaxUTRA | 21 (21 dBm) |  |  |
| cellReselectionPriority[n] | 5 | UTRA is of higher priority than E-UTRAN |  |
| } |  |  |  |

SystemInformationBlockType3: for inter-RAT EUTRAN FDD/TDD - UTRA FDD is of lower priority cell re-selection

Table H.2.3-5: SystemInformationBlockType3: Inter-RAT E-UTRAN FDD/TDD - UTRA FDD is of lower priority cell re-selection

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-2 SystemInformationBlockType3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 22 (44 dB) | 44 is actual value in dB (22 * 2 dB) |  |

SystemInformationBlockType6: for inter-RAT EUTRAN FDD/TDD - UTRA FDD is of lower priority cell re-selection

Table H.2.3-6: SystemInformationBlockType6: Inter-RAT E-UTRAN FDD/TDD - UTRA FDD is of lower priority cell re-selection

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-5 SystemInformationBlockType6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CarrierFreqListUTRA-FDD ::= SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| threshX-Low | 21 (42 dB) | 42 is actual value in dB (21 * 2 dB) |  |
| q-RxLevMin | -58 (-115 dBm) | -115 is actual value in dBm (-58 * 2 + 1 dBm) |  |
| p-MaxUTRA | 21 (21 dBm) |  |  |
| q-QualMin | -20 (-20 dB) |  |  |
| } |  |  |  |

SystemInformationBlockType3: for inter-RAT EUTRAN FDD/TDD - UTRA TDD is of lower priority cell re-selection

Table H.2.3-7: SystemInformationBlockType3: Inter-RAT E-UTRAN FDD/TDD - UTRA TDD is of lower priority cell re-selection

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-2 SystemInformationBlockType3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 23 (46 dB) | 46 is actual value in dB (23 * 2 dB) |  |

SystemInformationBlockType6: for inter-RAT EUTRAN FDD/TDD - UTRA TDD is of lower priority cell re-selection

Table H.2.3-8: SystemInformationBlockType6: Inter-RAT E-UTRAN FDD/TDD - UTRA TDD is of lower priority cell re-selection

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-5 SystemInformationBlockType6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CarrierFreqListUTRA-TDD ::= SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { |  |  | UTRA-TDD |
| threshX-Low | 12 (24 dB) | 24 is actual value in dB (12 * 2 dB) |  |
| q-RxLevMin | -52 (-103 dBm) | -103 is actual value in dBm (-52 * 2 + 1 dBm) |  |
| p-MaxUTRA | 21 (21 dBm) |  |  |
| } |  |  |  |

SystemInformationBlockType3: (FDD/TDD) for inter-RAT E-UTRAN - GSM cell re-selection

Table H.2.3-9: SystemInformationBlockType3: Inter-RAT E-UTRAN - GSM cell re-selection

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-2 SystemInformationBlockType3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearch | Not sent |  |  |
| threshServingLow | 22 (44 dB) | 44 is actual value in dB (22 * 2 dB) |  |
| cellReselectionPriority | 4 |  |  |

SystemInformationBlockType7: (FDD/TDD) for inter-RAT E-UTRAN - GSM cell re-selection

Table H.2.3-10: SystemInformationBlockType7: Inter-RAT E-UTRAN - GSM cell re-selection

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-6 SystemInformationBlockType7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| commonInfo SEQUENCE { |  |  |  |
| cellReselectionPriority | 0 |  |  |
| ncc-Permitted | ‘11111111'B |  |  |
| q-RxLevMin | 5 (-105 dBm) | -105 is actual value in dBm (5 * 2 - 115 dBm ) |  |
| p-MaxGERAN | 23 (23 dBm) |  | GSM 400 & GSM 900 & GSM 850 & GSM 700 |
|  | 24 (24 dBm) |  | DCS 1800 & PCS 1900 |
| threshX-High | 12 (24 dB) | 24 is actual value in dB (12 * 2 dB) |  |
| threshX-Low | 12 (24 dB) | 24 is actual value in dB (12 * 2 dB) |  |

SystemInformationBlockType3: Inter-RAT E-UTRAN FDD/TDD - HRPD is of lower priority cell re-selection

Table H.2.3-11: SystemInformationBlockType3: Inter-RAT E-UTRAN FDD/TDD - HRPD is of lower priority cell re-selection

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-2 SystemInformationBlockType3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearch | Not sent |  |  |
| threshServingLow | 22 (44 dB) | 44 is actual value in dB (22 * 2 dB) |  |
| cellReselectionPriority | 1 |  |  |
| } |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -70 (-140 dBm) | For RF/RRM test cases |  |
| s-IntraSearch | Not present |  |  |
| t-ReselectionEUTRA | 0 |  |  |

SystemInformationBlockType8: Inter-RAT E-UTRAN FDD/TDD - HRPD is of lower priority cell re-selection

Table H.2.3-12: SystemInformationBlockType8: Inter-RAT E-UTRAN FDD/TDD - HRPD is of lower priority cell re-selection

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-7 SystemInformationBlockType8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| cellReselectionParametersHRPD SEQUENCE { |  |  |  |
| bandClassList SEQUENCE (SIZE (1..maxCDMA -BandClass)) OF SEQUENCE { | 1 entry |  |  |
| cellReselectionPriority | 0 |  |  |
| threshX-High | 60(-30) | INTEGER (0..63) |  |
| threshX-Low | 28(-14) | INTEGER (0..63) |  |
| } |  |  |  |
| } |  |  |  |
| t-ReselectionCDMA2000 | 0 | INTEGER (0..7) |  |
| } |  |  |  |

System Information Block Type 3: for E-UTRAN to UTRAN inter-RAT cell re-selection

Table H.2.3-13: System Information Block type3: Inter-RAT E-UTRAN FDD/TDD - UTRAN FDD cell re-selection

| Derivation Path: 34.108 clause 6.1.0b |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| - Cell selection and reselection info |  |  |  |
| - Qqualmin | -20 |  |  |
| - Qrxlevlmin | -58 (-115dBm) |  |  |
| - Maximum allowed UL TX power | 21 |  |  |

Table H.2.3-14: System Information Block type 3 (1.28 Mcps TDD): inter-RAT E-UTRAN FDD/TDD - UTRAN TDD cell re-selection

| Derivation Path: 34.108 clause 6.1.0b |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| - SIB4 Indicator | TRUE |  |  |
| - Cell identity | 0000 0000 0000 0000 0000 0000 0001B |  |  |
| - Cell selection and re-selection info |  |  |  |
| - Mapping info | Not present |  |  |
| - Cell selection and reselection quality measure | (no data) |  |  |
| - CHOICE mode | TDD |  |  |
| - Sintrasearch | 10 dB |  |  |
| - Sintersearch | 10 dB |  |  |
| - SsearchHCS | Not present |  |  |
| - RAT List | Not present |  |  |
| - Qrxlevmin | -103 dBm |  |  |
| - Qhyst1s | 0 dB |  |  |
| - Treselections | 0 seconds |  |  |
| - HCS Serving cell information | Not present |  |  |

SystemInformationBlockType3: Inter-RAT E-UTRAN FDD/TDD - cdma2000 1xRTT is of lower priority cell re-selection

Table H.2.3-15: SystemInformationBlockType3: Inter-RAT E-UTRAN FDD/TDD - cdma2000 1xRTT is of lower priority cell re-selection

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-2 SystemInformationBlockType3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearch | Not sent |  |  |
| threshServingLow | 22 (44 dB) | 44 is actual value in dB (22 * 2 dB) |  |
| cellReselectionPriority | 1 |  |  |
| } |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -70 (-140 dBm) | For RF/RRM test cases |  |
| s-IntraSearch | Not present |  |  |
| t-ReselectionEUTRA | 0 |  |  |
| } |  |  |  |

SystemInformationBlockType8: Inter-RAT E-UTRAN FDD/TDD - cdma2000 1xRTT is of lower priority cell re-selection

Table H.2.3-16: SystemInformationBlockType8: Inter-RAT E-UTRAN FDD/TDD - cdma2000 1xRTT is of lower priority cell re-selection

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-7 SystemInformationBlockType8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| cellReselectionParameters1XRTT SEQUENCE { |  |  | 1XRTT |
| longCodeState1XRTT | Not Present |  |  |
| cellReselectionParameters1XRTT SEQUENCE { |  |  |  |
| bandClassList SEQUENCE (SIZE (1..maxCDMA -BandClass)) OF SEQUENCE { | 1 entry |  |  |
| cellReselectionPriority | 0 |  |  |
| threshX-High | 60(-30) | INTEGER (0..63) |  |
| threshX-Low | 56(-28) | INTEGER (0..63) |  |
| } |  |  |  |
| t-ReselectionCDMA2000 | 0 | INTEGER (0..7) |  |
| t-ReselectionCDMA2000-SF | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

## H.2.4	System information blocks message contents exceptions for E-UTRAN radio link monitoring (RLM)

SystemInformationBlockType2: (FDD/TDD) for E-UTRAN Radio Link Monitoring test for out-of-sync

Table H.2.4-1: SystemInformationBlockType2: E-UTRAN Radio Link Monitoring test for out-of-sync

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 SystemInformationBlockType2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType 2 ::= SEQUENCE { |  |  |  |
| radioResourceConfigCommon SEQUENCE { |  |  |  |
| pdsch-ConfigCommon SEQUENCE { |  |  |  |
| referenceSignalPower | Set to an arbitrarily selected value above -11dBm and within the IE allowed range described in 3GPP TS 36.331 [5] | The selected IE value depends on the test system implementation and should be declared in the test report. |  |
| } |  |  |  |
| soundingRS-UL-ConfigCommon CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| ue-TimersAndConstants { |  |  |  |
| t300 | ms1000 |  |  |
| t301 | ms1000 |  |  |
| t310 | ms0 |  |  |
| n310 | n1 |  |  |
| t311 | ms1000 |  |  |
| n311 | n1 |  |  |
| } |  |  |  |

SystemInformationBlockType2: (FDD/TDD) for E-UTRAN Radio Link Monitoring test for in-sync

Table H.2.4-2: SystemInformationBlockType2: E-UTRAN Radio Link Monitoring test for in-sync

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-2 SystemInformationBlockType2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType 2 ::= SEQUENCE { |  |  |  |
| radioResourceConfigCommon SEQUENCE { |  |  |  |
| pdsch-ConfigCommon SEQUENCE { |  |  |  |
| referenceSignalPower | Set to an arbitrarily selected value above -11dBm and within the IE allowed range described in 3GPP TS 36.331 [5] | The selected IE value depends on the test system implementation and should be declared in the test report. |  |
| } |  |  |  |
| soundingRS-UL-ConfigCommon CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| ue-TimersAndConstants { |  |  |  |
| t300 | ms1000 |  |  |
| t301 | ms1000 |  |  |
| t310 | ms2000 |  |  |
| n310 | n1 |  |  |
| t311 | ms1000 |  |  |
| n311 | n1 |  |  |
| } |  |  |  |

## H.2.5	System information blocks message contents exceptions for RRC Re-establishment

SystemInformationBlockType2: (FDD/TDD) for E-UTRAN Intra-frequency RRC Re-establishment

Table H.2.5-1: SystemInformationBlockType2: E-UTRAN FDD Intra-frequency RRC Re-establishment

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 SystemInformationBlockType2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType 2 ::= SEQUENCE { |  |  |  |
| ue-TimersAndConstants { |  |  |  |
| t310 | ms0 |  |  |
| t311 | ms3000 |  |  |
| n310 | n1 |  |  |
| n311 | n1 |  |  |

SystemInformationBlockType2: (FDD/TDD) for E-UTRAN Inter-frequency RRC Re-establishment

Table H.2.5-2: SystemInformationBlockType2: E-UTRAN FDD Inter-frequency RRC Re-establishment

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 SystemInformationBlockType2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType 2 ::= SEQUENCE { |  |  |  |
| ue-TimersAndConstants { |  |  |  |
| t310 | ms0 |  |  |
| t311 | ms5000 |  |  |
| n310 | n1 |  |  |
| n311 | n1 |  |  |

## H.2.6	System information block messages and information elements contents exceptions for E-UTRAN Random Access

SystemInformationBlockType1: (FDD/TDD) for E-UTRAN random access

Table H.2.6-1: SystemInformationBlockType1: E-UTRAN FDD/TDD - Contention and Non-Contention Based Random Access

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.2, Table 4.4.3.2-3 SystemInformationBlockType1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| p-Max | 23 (dBm) |  |  |

Table H.2.6-1a: SystemInformationBlockType1-BR-r13: E-UTRAN FDD/TDD - Contention and Non-Contention Based Random Access

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.2, Table 4.4.3.2-3A SystemInformationBlockType1-BR-r13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| p-Max | 23 (dBm) |  |  |

RACH-ConfigCommon-DEFAULT: (FDD/TDD) for E-UTRAN random access

Table H.2.6-2: RACH-ConfigCommon-DEFAULT: E-UTRAN FDD/TDD - Contention and Non-Contention Based Random Access

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.3, Table 4.6.3-12 RACH-ConfigCommon-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| preambleInfo SEQUENCE { |  |  |  |
| numberOfRA-Preambles | n52 |  |  |
| preamblesGroupAConfig SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| powerRampingParameters SEQUENCE { |  |  |  |
| powerRampingStep | dB2 |  |  |
| preambleInitialReceivedTargetPower | dBm-120 |  |  |
| } |  |  |  |
| ra-SupervisionInfo SEQUENCE { |  |  | Cat-M1 UE |
| preambleTransMax | n6 |  |  |
| } |  |  |  |
| maxHARQ-Msg3Tx | 4 |  |  |
| } |  |  |  |
| RAR-HoppingConfigInfo-r13 [1] SEQUENCE { |  |  |  |
| rar-HoppingConfig | off |  |  |
| } |  |  |  |

PDSCH-ConfigCommon-DEFAULT: (FDD/TDD) for E-UTRAN random access

Table H.2.6-3: PDSCH-ConfigCommon-DEFAULT: E-UTRAN FDD/TDD - Contention and Non-Contention Based Random Access

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.3, Table 4.6.3-5 PDSCH-ConfigCommon-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| referenceSignalPower | -5 (dBm) |  | 1TX |

## H.2.7	System information blocks message contents exceptions for eICIC/feICIC

SystemInformationBlockType2: (FDD/TDD) for eICIC/feICIC

Table H.2.7-1: SystemInformationBlockType2: Time Domain Measurement Resource Restriction with Non MBSFN ABS (eICIC/feICIC)

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 SystemInformationBlockType2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| mbsfn-SubframeConfiguration | Not present |  |  |
| } |  |  |  |

SystemInformationBlockType3: (FDD) for eICIC/feICIC

Table H.2.7-2: SystemInformationBlockType3: Time Domain Measurement Resource Restriction with Non MBSFN ABS (eICIC/feICIC)

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-2 SystemInformationBlockType3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| neighCellConfig | '01'B (No MBSFN subframes are present in all neighbour cells) |  | FDD with E-UTRA FDD neighbour cell |

## H.2.8	System information blocks message contents exceptions for eMTC

SystemInformationBlockType1-BR-r13: (FDD/TDD) for eMTC.

Table H.2.8-1: SystemInformationBlockType1-BR-r13

| Derivation Path: 3GPP TS 36.508 clause 4.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| freqHoppingParametersDL-r13 SEQUENCE { |  |  |  |
| mpdcch-pdsch-HoppingNB-r13 | nb2 | 2 narrowbands |  |
| interval-DLHoppingConfigCommonModeA-r13 CHOICE { |  |  |  |
| interval-FDD-r13 | int4 |  | FDD |
| interval-TDD-r13 | Int10 |  | TDD |
| } |  |  |  |
| interval-DLHoppingConfigCommonModeB-r13 CHOICE { |  |  |  |
| interval-FDD-r13 | int8 |  | FDD |
| interval-TDD-r13 | int20 |  | TDD |
| } |  |  |  |
| mpdcch-pdsch-HoppingOffset-r13 | 7 | INTEGER (1..16 (maxAvailNarrowBands-r13)) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

## H.2.9	System information blocks message contents exceptions for HD-FDD radio link monitoring (RLM) for UE Category NB1

SystemInformationBlockType2-NB: for HD-FDD Radio Link Monitoring test for out-of-sync for UE category NB1

Table H.2.9-1: SystemInformationBlockType2-NB: HD-FDD Radio Link Monitoring test for out-of-sync for UE category NB1

| Derivation Path: 3GPP TS 36.331 clause 8.1.4.3.3, Table 8.1.4.3.3-1 SystemInformationBlockType2-NB1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2-NB-r13 ::= SEQUENCE { |  |  |  |
| ue-TimersAndConstants-r13 SEQUENCE { |  |  |  |
| t310-r13 | ms0 |  |  |
| n310-r13 | n1 |  |  |
| t311-r13 | ms1000 |  |  |
| n311-r13 | n1 |  |  |
| } |  |  |  |
| } |  |  |  |

SystemInformationBlockType2-NB: for HD-FDD Radio Link Monitoring test for in-sync for UE category NB1

Table H.2.9-2: SystemInformationBlockType2-NB: HD-FDD Radio Link Monitoring test for in-sync for UE category NB1

| Derivation Path: 3GPP TS 36.331 clause 8.1.4.3.3, Table 8.1.4.3.3-1 SystemInformationBlockType2-NB1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2-NB-r13 ::= SEQUENCE { |  |  |  |
| ue-TimersAndConstants-r13 SEQUENCE { |  |  |  |
| t310-r13 | Ms4000 |  |  |
| n310-r13 | n1 |  |  |
| t311-r13 | ms1000 |  |  |
| n311-r13 | n1 |  |  |
| } |  |  |  |
| } |  |  |  |

# H.3	Default RRC messages and information elements contents exceptions

This clause contains the default values of common sRRC messages and information elements, other than those described in 3GPP TS 36.508 [7].

## H.3.1	RRC messages and information elements contents exceptions for E-UTRAN measurement configuration

RRCConnectionReconfiguration: (FDD/TDD) to setup E-UTRAN Measurement Configuration

Table H.3.1-1: RRCConnectionReconfiguration: E-UTRAN Measurement Configuration

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.1, Table 4.6.1-8 RRCConnectionReconfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | MeasConfig -DEFAULT |  | MEAS |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

MeasConfig-DEFAULT (FDD/TDD) E-UTRAN measurement configuration for intra frequency measurement

Table H.3.1-2: MeasConfig-DEFAULT: E-UTRAN intra frequency measurement configuration

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-1 MeasConfig-DEFAULT: |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId | IdMeasObject-f1 | f1 is the frequency of the serving cell |  |
| measObject CHOICE { |  |  |  |
| MeasObjectEUTRA | MeasObjectEUTRA-GENERIC(f1) |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId) )OF SEQUENCE { | 1 entry |  |  |
| reportConfigId | idReportConfig-A3 |  |  |
| reportConfig | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) of SEQUENCE { | 1 entry |  |  |
| measId | 1 |  |  |
| measObjectId | IdMeasObject-f1 |  |  |
| reportConfigId | idReportConfig-A3 |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig-DEFAULT |  |  |
| measGapConfig | Not present |  |  |
| s-Measure | Not present |  |  |
| preRegistrationInfoHRPD | Not present |  |  |
| speedStatePars | Not present |  |  |
| } |  |  |  |

Table H.3.1-2a: MeasConfig-DEFAULT: E-UTRAN intra frequency measurement configuration for Cat-M1 UE

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-1 MeasConfig-DEFAULT: |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId | IdMeasObject-f1 | f1 is the frequency of the serving cell |  |
| measObject CHOICE { |  |  |  |
| MeasObjectEUTRA | MeasObjectEUTRA-GENERIC(f1) |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId) )OF SEQUENCE { | 1 entry |  |  |
| reportConfigId | idReportConfig-A3 |  |  |
| reportConfig | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) of SEQUENCE { | 1 entry |  |  |
| measId | 1 |  |  |
| measObjectId | IdMeasObject-f1 |  |  |
| reportConfigId | idReportConfig-A3 |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig-DEFAULT |  |  |
| measGapConfig | MeasGapConfig-GP1 |  | Gap Pattern Id = 0 |
|  | MeasGapConfig-GP2 |  | Gap Pattern Id = 1 |
| s-Measure | Not present |  |  |
| preRegistrationInfoHRPD | Not present |  |  |
| speedStatePars | Not present |  |  |
| } |  |  |  |

MeasConfig-DEFAULT (FDD/TDD) E-UTRAN measurement configuration for inter frequency handover

Table H.3.1-3: MeasConfig-DEFAULT: E-UTRAN inter frequency measurement configuration for inter frequency handover

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-1 MeasConfig-DEFAULT: |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entry |  |  |
| MeasObjectToAddMod SEQUENCE { |  | f1 is the frequency of the serving cell |  |
| measObjectId | IdMeasObject-f1 |  |  |
| measObject CHOICE { |  |  |  |
| MeasObjectEUTRA | MeasObjectEUTRA-GENERIC(f1) |  |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f2 | f2 is the frequency of the neighbouring cell(inter frequency cell) |  |
| measObject CHOICE { |  |  |  |
| MeasObjectEUTRA | MeasObjectEUTRA-GENERIC(f2) | inter frequency cell |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId) )OF SEQUENCE { | 1 entry |  |  |
| reportConfigId | idReportConfig-A3 |  |  |
| reportConfig | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) of SEQUENCE { | 1 entry |  |  |
| measId | 1 |  |  |
| measObjectId | IdMeasObject-f2 |  |  |
| reportConfigId | idReportConfig-A3 |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig-DEFAULT |  |  |
| measGapConfig | MeasGapConfig-GP1 |  | Gap Pattern Id = 0 |
|  | MeasGapConfig-GP2 |  | Gap Pattern Id = 1 |
| s-Measure | Not present |  |  |
| preRegistrationInfoHRPD | Not present |  |  |
| speedStatePars | Not present |  |  |
| } |  |  |  |

MeasConfig-DEFAULT (FDD/TDD) E-UTRAN measurement configuration for E-UTRAN to UTRAN cell search

Table H.3.1-4: MeasConfig-DEFAULT: interRAT UTRAN measurement configuration for E-UTRAN to UTRAN cell search

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-1 MeasConfig-DEFAULT: |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entry |  |  |
| MeasObjectToAddMod SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f1 | f1 is the frequency of the serving cell (E-UTRA Cell) |  |
| measObject CHOICE { |  |  |  |
| MeasObjectEUTRA | MeasObjectEUTRA-GENERIC(f1) | E-UTRA Cell |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f2 | f2 is the frequency of the neighbouring cell(UTRA Cell) |  |
| measObject CHOICE { |  |  |  |
| measObjectUTRA | MeasObjectUTRA-GENERIC(f2) | UTRA Cell |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId) )OF SEQUENCE { | 1 entry |  |  |
| reportConfigId | idReportConfig-B1 |  |  |
| reportConfig | ReportConfigInterRAT-B1-UTRA |  |  |
| } |  |  |  |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) of SEQUENCE { | 1 entry |  |  |
| measId | 1 |  |  |
| measObjectId | IdMeasObject-f2 |  |  |
| reportConfigId | idReportConfig-B1 |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig-DEFAULT |  | UTRAN |
| measGapConfig | MeasGapConfig-GP1 |  | Gap Pattern Id = 0 |
|  | MeasGapConfig-GP2 |  | Gap Pattern Id = 1 |
| s-Measure | Not present |  |  |
| preRegistrationInfoHRPD | Not present |  |  |
| speedStatePars | Not present |  |  |
| } |  |  |  |

MeasConfig-DEFAULT (FDD/TDD) E-UTRAN measurement configuration for E-UTRAN to UTRAN handover

Table H.3.1-5: MeasConfig-DEFAULT: interRAT UTRAN measurement configuration for E-UTRAN to UTRAN handover

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-1 MeasConfig-DEFAULT: |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entry |  |  |
| MeasObjectToAddMod SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f1 | f1 is the frequency of the serving cell(E-UTRA Cell) |  |
| measObject CHOICE { |  |  |  |
| MeasObjectEUTRA | MeasObjectEUTRA-GENERIC(f1) | E-UTRA Cell |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f2 | f2 is the frequency of the neighbouring cell(UTRA Cell) |  |
| measObject CHOICE { |  |  |  |
| measObjectUTRA | MeasObjectUTRA-GENERIC(f2) | UTRA Cell |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId | idReportConfig-B2 |  |  |
| reportConfig | ReportConfigInterRAT-B2-UTRA |  |  |
| } |  |  |  |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId | 1 |  |  |
| measObjectId | IdMeasObject-f2 |  |  |
| reportConfigId | IdReportConfig-B2 |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig-DEFAULT |  | UTRAN |
| measGapConfig | MeasGapConfig-GP1 |  | Gap Pattern Id = 0 |
|  | MeasGapConfig-GP2 |  | Gap Pattern Id = 1 |
| s-Measure | Not present |  |  |
| preRegistrationInfoHRPD | Not present |  |  |
| speedStatePars | Not present |  |  |
| } |  |  |  |

MeasConfig-DEFAULT (FDD/TDD) E-UTRAN measurement configuration for E-UTRAN to GSM handover

Table H.3.1-6: MeasConfig-DEFAULT: interRAT GSM measurement configuration for E-UTRAN to GSM handover

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-1 MeasConfig-DEFAULT: |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entry |  |  |
| MeasObjectToAddMod SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f1 | f1 is the frequency of the serving cell(E-UTRA Cell) |  |
| measObject CHOICE { |  |  |  |
| MeasObjectEUTRA | MeasObjectEUTRA-GENERIC(f1) | E-UTRA Cell |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f2 | f2 is the frequency of the neighbouring cell(GERAN Cell) |  |
| measObject CHOICE { |  |  |  |
| measObjectGERAN | MeasObjectGERAN-GENERIC(f2) | GERAN Cell |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId) )OF SEQUENCE { | 1 entry |  |  |
| reportConfigId | idReportConfig-B1 |  |  |
| reportConfig | ReportConfigInterRAT-B1-GERAN |  |  |
| } |  |  |  |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) of SEQUENCE { | 1 entry |  |  |
| measId | 1 |  |  |
| measObjectId | IdMeasObject-f2 |  |  |
| reportConfigId | idReportConfig-B1 |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig-DEFAULT |  | GERAN |
| measGapConfig | MeasGapConfig-GP1 |  | Gap Pattern Id = 0 |
|  | MeasGapConfig-GP2 |  | Gap Pattern Id = 1 |
| s-Measure | Not present |  |  |
| preRegistrationInfoHRPD | Not present |  |  |
| speedStatePars | Not present |  |  |
| } |  |  |  |

QuantityConfig-DEFAULT (FDD/TDD) E-UTRAN measurement configuration for L3 filtering is not used

Table H.3.1-7: QuantityConfig-DEFAULT: measurement configuration for L3 filtering is not used

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-3A: QuantityConfig-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QuantityConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| quantityConfigEUTRA SEQUENCE { |  |  |  |
| filterCoefficientRSRP | fc0 |  |  |
| filterCoefficientRSRQ | fc0 |  |  |
| } |  |  |  |
| quantityConfigUTRA SEQUENCE {} | Not present |  |  |
| quantityConfigUTRA SEQUENCE { |  |  | UTRAN |
| measQuantityUTRA-FDD | cpich-EcN0 |  |  |
| measQuantityUTRA-TDD | pccpch-RSCP |  |  |
| filterCoefficient | fc0 |  |  |
| } |  |  |  |
| quantityConfigGERAN SEQUENCE {} | Not present |  |  |
| quantityConfigGERAN SEQUENCE { |  |  | GERAN |
| measQuantityGERAN | rssi |  |  |
| filterCoefficient | fc0 |  |  |
| } |  |  |  |
| quantityConfigCDMA2000 SEQUENCE {} | Not present |  |  |
| quantityConfigCDMA2000 SEQUENCE { |  |  | CDMA2000 |
| measQuantityCDMA2000 | pilotStrength |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRAN | For inter-RAT measurements with UTRAN |
| GERAN | For inter-RAT measurements with GERAN |
| CDMA2000 | For inter-RAT measurements with CDMA2000 |

Table H.3.1-8: MeasConfig-DEFAULT: interRAT HRPD measurement configuration for E-UTRAN to HRPD handover

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-1 MeasConfig-DEFAULT: |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entry |  |  |
| MeasObjectToAddMod SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f1 | f1 is the frequency of the serving cell(E-UTRA Cell) |  |
| measObject CHOICE { |  |  |  |
| measObjectEUTRA | MeasObjectEUTRA-GENERIC(f1) | E-UTRA Cell |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f2 | f2 is the frequency of the neighbouring cell(CDMA2000 Cell) |  |
| measObject CHOICE { |  |  |  |
| measObjectCDMA2000 | MeasObjectCDMA2000 | CDMA2000 Cell |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId | idReportConfig-B2 |  |  |
| reportConfig | ReportConfigInterRAT-B2-CDMA2000 |  |  |
| } |  |  |  |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId | 1 |  |  |
| measObjectId | IdMeasObject-f2 |  |  |
| reportConfigId | IdReportConfig-B2 |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig-DEFAULT |  | CDMA2000 |
| measGapConfig | MeasGapConfig-GP1 |  | Gap Pattern Id = 0 |
| s-Measure | Not present |  |  |
| preRegistrationInfoHRPD | Not present |  |  |
| speedStatePars | Not present |  |  |
| } |  |  |  |

MeasConfig-DEFAULT (FDD/TDD) E-UTRAN measurement configuration for inter frequency measurement

Table H.3.1-9: MeasConfig-DEFAULT: E-UTRAN inter frequency measurement configuration

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-1 MeasConfig-DEFAULT: |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entry |  |  |
| MeasObjectToAddMod SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f1 | f1 is the frequency of the serving cell |  |
| measObject CHOICE { |  |  |  |
| MeasObjectEUTRA | MeasObjectEUTRA-GENERIC(f1) |  |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f2 | f2 is the frequency of the neighbouring cell(inter frequency cell) |  |
| measObject CHOICE { |  |  |  |
| MeasObjectEUTRA | MeasObjectEUTRA-GENERIC(f2) | inter frequency cell |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId) )OF SEQUENCE { | 1 entry |  |  |
| reportConfigId | idReportConfig-A3 |  |  |
| reportConfig | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) of SEQUENCE { | 1 entry |  |  |
| measId | 1 |  |  |
| measObjectId | IdMeasObject-f2 |  |  |
| reportConfigId | idReportConfig-A3 |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig-DEFAULT |  |  |
| measGapConfig | MeasGapConfig-GP1 |  | Gap Pattern Id = 0 |
|  | MeasGapConfig-GP2 |  | Gap Pattern Id = 1 |
| s-Measure | Not present |  |  |
| preRegistrationInfoHRPD | Not present |  |  |
| speedStatePars | Not present |  |  |
| } |  |  |  |

MeasConfig-DEFAULT (FDD/TDD) E-UTRAN measurement configuration for E-UTRAN to GSM cell search

Table H.3.1-10: MeasConfig-DEFAULT: interRAT GSM measurement configuration for E-UTRAN to GSM cell search

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-1 MeasConfig-DEFAULT: |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 7 entry |  |  |
| MeasObjectToAddMod SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f1 | f1 is the frequency of the serving cell(E-UTRA Cell) |  |
| measObject CHOICE { |  |  |  |
| MeasObjectEUTRA | MeasObjectEUTRA-GENERIC(f1) | E-UTRA Cell |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f2 | f2 is the frequency of the neighbouring cell(GERAN Cell) |  |
| measObject CHOICE { |  |  |  |
| measObjectGERAN | MeasObjectGERAN-GENERIC(f2) | GERAN Cell |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f3 | f3 is the frequency of the neighbouring cell(GERAN Cell) |  |
| measObject CHOICE { |  |  |  |
| measObjectGERAN | MeasObjectGERAN-GENERIC(f3) | GERAN Cell |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f4 | f4 is the frequency of the neighbouring cell(GERAN Cell) |  |
| measObject CHOICE { |  |  |  |
| measObjectGERAN | MeasObjectGERAN-GENERIC(f4) | GERAN Cell |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f5 | f5 is the frequency of the neighbouring cell(GERAN Cell) |  |
| measObject CHOICE { |  |  |  |
| measObjectGERAN | MeasObjectGERAN-GENERIC(f5) | GERAN Cell |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f6 | f6 is the frequency of the neighbouring cell(GERAN Cell) |  |
| measObject CHOICE { |  |  |  |
| measObjectGERAN | MeasObjectGERAN-GENERIC(f6) | GERAN Cell |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f7 | f7 is the frequency of the neighbouring cell(GERAN Cell) |  |
| measObject CHOICE { |  |  |  |
| measObjectGERAN | MeasObjectGERAN-GENERIC(f7) | GERAN Cell |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId) )OF SEQUENCE { | 1 entry |  |  |
| reportConfigId | idReportConfig-B1 |  |  |
| reportConfig | ReportConfigInterRAT-B1-GERAN |  |  |
| } |  |  |  |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) of SEQUENCE { | 1 entry |  |  |
| measId | 1 |  |  |
| measObjectId | IdMeasObject-f2 |  |  |
| reportConfigId | idReportConfig-B1 |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig-DEFAULT |  | GERAN |
| measGapConfig | MeasGapConfig-GP1 |  | Gap Pattern Id = 0 |
|  | MeasGapConfig-GP2 |  | Gap Pattern Id = 1 |
| s-Measure | Not present |  |  |
| preRegistrationInfoHRPD | Not present |  |  |
| speedStatePars | Not present |  |  |
| } |  |  |  |

MeasConfig-DEFAULT (FDD/TDD) E-UTRAN measurement periodical configuration for E-UTRAN to GSM cell search

Table H.3.1-11: MeasConfig-DEFAULT: interRAT GSM measurement periodical configuration for E-UTRAN to GSM cell search

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-1 MeasConfig-DEFAULT: |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 7 entry |  |  |
| MeasObjectToAddMod SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f1 | f1 is the frequency of the serving cell(E-UTRA Cell) |  |
| measObject CHOICE { |  |  |  |
| MeasObjectEUTRA | MeasObjectEUTRA-GENERIC(f1) | E-UTRA Cell |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f2 | f2 is the frequency of the neighbouring cell(GERAN Cell) |  |
| measObject CHOICE { |  |  |  |
| measObjectGERAN | MeasObjectGERAN-GENERIC(f2) | GERAN Cell |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f3 | f3 is the frequency of the neighbouring cell(GERAN Cell) |  |
| measObject CHOICE { |  |  |  |
| measObjectGERAN | MeasObjectGERAN-GENERIC(f3) | GERAN Cell |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f4 | f4 is the frequency of the neighbouring cell(GERAN Cell) |  |
| measObject CHOICE { |  |  |  |
| measObjectGERAN | MeasObjectGERAN-GENERIC(f4) | GERAN Cell |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f5 | f5 is the frequency of the neighbouring cell(GERAN Cell) |  |
| measObject CHOICE { |  |  |  |
| measObjectGERAN | MeasObjectGERAN-GENERIC(f5) | GERAN Cell |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f6 | f6 is the frequency of the neighbouring cell(GERAN Cell) |  |
| measObject CHOICE { |  |  |  |
| measObjectGERAN | MeasObjectGERAN-GENERIC(f6) | GERAN Cell |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f7 | f7 is the frequency of the neighbouring cell(GERAN Cell) |  |
| measObject CHOICE { |  |  |  |
| measObjectGERAN | MeasObjectGERAN-GENERIC(f7) | GERAN Cell |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId) )OF SEQUENCE { | 1 entry |  |  |
| reportConfigId | idReportConfig-P |  |  |
| reportConfig | ReportConfigInterRAT-PERIODICAL |  |  |
| } |  |  |  |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) of SEQUENCE { | 2 entry |  |  |
| measIdToAddMod ::= SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measObjectId | IdMeasObject-f2 |  |  |
| reportConfigId | idReportConfig-P |  |  |
| } |  |  |  |
| measIdToAddMod ::= SEQUENCE { |  |  |  |
| measId | 2 |  |  |
| measObjectId | IdMeasObject-f3 |  |  |
| reportConfigId | idReportConfig-P |  |  |
| } |  |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig-DEFAULT |  | GERAN |
| measGapConfig | MeasGapConfig-GP1 |  | Gap Pattern Id = 0 |
|  | MeasGapConfig-GP2 |  | Gap Pattern Id = 1 |
| s-Measure | Not present |  |  |
| preRegistrationInfoHRPD | Not present |  |  |
| speedStatePars | Not present |  |  |
| } |  |  |  |

## H.3.2	RRC messages and information elements contents exceptions for E-UTRAN cell re-selection and handover

PRACH-Config-DEFAULT: (FDD) for cell re-selection and handover

Table H.3.2-1: PRACH-Config-DEFAULT: E-UTRAN FDD cell re-selection and handover

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.3, Table 4.6.3-7 PRACH-Config-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PRACH-ConfigInfo SEQUENCE { |  |  |  |
| prach-ConfigIndex | 4 |  |  |

PRACH-Config-DEFAULT: (TDD) for cell re-selection and intra frequency / inter frequency handover

Table H.3.2-2: PRACH-Config-DEFAULT: E-UTRAN TDD cell re-selection and handover

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.3, Table 4.6.3-7 PRACH-Config-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PRACH-ConfigInfo SEQUENCE { |  |  |  |
| prach-ConfigIndex | 53 |  |  |

RRCConnectionReconfiguration: (FDD/TDD) for intra-frequency / inter-frequency handover

Table H.3.2-3: RRCConnectionReconfiguration: E-UTRAN handover Configuration

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.1, Table 4.6.1-8 RRCConnectionReconfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| mobilityControlInfo |  |  |  |
| } | MobilityControlInfo-HO |  | HO |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

## H.3.3	RRC messages and information elements contents exceptions for E-UTRAN inter-RAT handover

Handover: (FDD/TDD) to perform Inter-RAT E-UTRAN - UTRAN handover

Table H.3.3-1: Handover: Inter-RAT E-UTRAN - UTRAN handover

| Derivation Path: 3GPP TS 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Handover ::= SEQUENCE { |  |  |  |
| targetRAT-Type | utra | ENUMERATED {utra, geran, cdma2000-1XTT, cdma2000-HRPD, spare4, spare3, spare2, spare1, …} |  |
| targetRAT-MessageContainer |  | OCTET STRING |  |
| nas-SecurityParamFromEUTRA |  | OCTET STRING(SIZE (1)) | UTRAGERAN |
| } |  |  |  |

Handover: (FDD/TDD) to perform Inter-RAT E-UTRAN - GSM handover

Table H.3.3-2: Handover: Inter-RAT E-UTRAN - GSM handover

| Derivation Path: 3GPP TS 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Handover ::= SEQUENCE { |  |  |  |
| targetRAT-Type | geran | ENUMERATED {utran, geran, cdma2000-1XTT, cdma2000-HRPD, spare4, spare3, spare2, spare1, …} |  |
| targetRAT-MessageContainer |  | OCTET STRING |  |
| nas-SecurityParamFromEUTRA |  | OCTET STRING(SIZE (1)) | UTRAGERAN |
| } |  |  |  |

MobilityFromEUTRACommand: (FDD/TDD) to setup Inter-RAT E-UTRAN handover

Table H.3.3-3: MobilityFromEUTRACommand: Inter-RAT E-UTRAN handover

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.1, Table 4.6.1-6 MobilityFromEUTRACommand |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityFromEUTRACommand ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| mobilityFromEUTRACommand-r8 SEQUENCE { |  |  |  |
| csFallbackIndicator | FALSE |  |  |
| purpose CHOICE { |  |  |  |
| Handover | Handover |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  | GERAN |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| bandIndicator |  | ENUMERATED {dcs1800, pcs1900} |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GERAN | The field should be present if the purpose is set to "handover" and the targetRAT-Type is set to "geran"; otherwise the field is not present |

Handover: (FDD/TDD) to perform Inter-RAT E-UTRAN - HRPD handover

Table H.3.3-4: Handover: Inter-RAT E-UTRAN - HRPD handover

| Derivation Path: 3GPP TS 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Handover ::= SEQUENCE { |  |  |  |
| targetRAT-Type | cdma2000-HRPD | ENUMERATED {utran, geran, cdma2000-1XTT, cdma2000-HRPD, spare4, spare3, spare2, spare1, …} |  |

## H.3.4	RRC messages and information elements exceptions for E-UTRAN UE transmit timing accuracy and UE timing advance adjustment accuracy

RRCConnectionReconfiguration: (FDD/TDD) to establish E-UTRAN Radio Resource Configuration

Table H.3.4-1: RRCConnectionReconfiguration: E-UTRAN Radio Resource Configuration

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.1, Table 4.6.1-8 RRCConnectionReconfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| Rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| C1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated |  |  |  |
|  | RadioResourceConfigDedicated-HO-TO-EUTRA(n,m) |  | HO-TO-EUTRA(n,m) |

PhysicalConfigDedicated-DEFAULT: (FDD/TDD) for E-UTRAN Physical Configuration

Table H.3.4-2: PhysicalConfigDedicated-DEFAULT: E-UTRAN Physical Configuration

| Derivation Path: 3GPP TS 36.508 [7] clause 4.8.21.6, Table 4.8.2.1.6-1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| soundingRS-UL-ConfigDedicated |  |  | SRB1 |
|  | SoundingRS-UL-ConfigDedicated-DEFAULT |  | RBC |
| antennaInformation CHOICE { |  |  |  |
| defaultValue | NULL |  |  |
| } |  |  |  |
| schedulingRequestConfig | Not present |  | SRB1 |
|  | SchedulingRequest-Config-DEFAULT |  | SRB1 AND (CEModeA OR CEModeB) |

## H.3.5	RRC messages and information elements contents exceptions for E-UTRAN RSRP and RSRQ Accuracy

MeasConfig: (FDD/TDD) to perform Measurement Configuration for E-UTRAN intra frequency RSRP and RSRQ accuracy

Table H.3.5-1: MeasConfig-DEFAULT: E-UTRAN intra frequency RSRP and RSRQ Accuracy

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-1 MeasConfig-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId | IdMeasObject-f1 | f1 is the frequency of the serving cell |  |
| measObject CHOICE { |  |  |  |
| MeasObjectEUTRA | MeasObjectEUTRA-GENERIC(f1) |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId) )OF SEQUENCE { | 1 entry |  |  |
| reportConfigId | idReportConfig-P |  |  |
| reportConfig | ReportConfigEUTRA- PERIODICAL |  |  |
| } |  |  |  |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) of SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measObjectId | IdMeasObject-f1 |  |  |
| reportConfigId | idReportConfig-P |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig-DEFAULT |  |  |
| measGapConfig | Not present |  |  |
| s-Measure | Not present |  |  |
| preRegistrationInfoHRPD | Not present |  |  |
| speedStatePars | Not present |  |  |
| } |  |  |  |

Table H.3.5-1a: MeasConfig-DEFAULT: E-UTRAN intra frequency RSRP and RSRQ Accuracy for Cat-M1 UE

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-1 MeasConfig-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId | IdMeasObject-f1 | f1 is the frequency of the serving cell |  |
| measObject CHOICE { |  |  |  |
| MeasObjectEUTRA | MeasObjectEUTRA-GENERIC(f1) |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId) )OF SEQUENCE { | 1 entry |  |  |
| reportConfigId | idReportConfig-P |  |  |
| reportConfig | ReportConfigEUTRA- PERIODICAL |  |  |
| } |  |  |  |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) of SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measObjectId | IdMeasObject-f1 |  |  |
| reportConfigId | idReportConfig-P |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig-DEFAULT |  |  |
| measGapConfig | MeasGapConfig-GP1 |  | Gap Pattern Id = 0 |
|  | MeasGapConfig-GP2 |  | Gap Pattern Id = 1 |
| s-Measure | Not present |  |  |
| preRegistrationInfoHRPD | Not present |  |  |
| speedStatePars | Not present |  |  |
| } |  |  |  |

MeasConfig: (FDD/TDD) perform Measurement Configuration for E-UTRAN inter frequency RSRP and RSRQ accuracy

Table H.3.5-2: MeasConfig-DEFAULT: E-UTRAN inter frequency RSRP and RSRQ Accuracy

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-1 MeasConfig-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entry |  |  |
| MeasObjectToAddMod SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f1 | f1 is the frequency of the serving cell |  |
| measObject CHOICE { |  |  |  |
| MeasObjectEUTRA | MeasObjectEUTRA-GENERIC(f1) |  |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f2 | f2 is the frequency of the neighbouring cell (inter frequency cell) |  |
| measObject CHOICE { |  |  |  |
| MeasObjectEUTRA | MeasObjectEUTRA-GENERIC(f2) | inter frequency cell |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId) )OF SEQUENCE { | 1 entry |  |  |
| reportConfigId | idReportConfig-P |  |  |
| reportConfig | ReportConfigEUTRA- PERIODICAL |  |  |
| } |  |  |  |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) of SEQUENCE { | 1 entry |  |  |
| measId | 1 |  |  |
| measObjectId | IdMeasObject-f2 |  |  |
| reportConfigId | idReportConfig-P |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig-DEFAULT |  |  |
| measGapConfig | MeasGapConfig-GP1 |  | Gap Pattern Id = 0 |
|  | MeasGapConfig-GP2 |  | Gap Pattern Id = 1 |
| s-Measure | Not present |  |  |
| preRegistrationInfoHRPD | Not present |  |  |
| speedStatePars | Not present |  |  |
| } |  |  |  |

ReportConfigEUTRA-PEROIDICAL: (FDD/TDD) for periodical configuration reporting of E-UTRAN RSRP accuracy

Table H.3.5-3: ReportConfigEUTRA-PERIODICAL: E-UTRAN RSRP Accuracy

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-7 ReportConfigEUTRA-PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-PERIODICAL ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| purpose | reportStrongestCells |  |  |
| } |  |  |  |
| } |  |  |  |
| triggerQuantity | rsrp |  |  |
| reportQuantity | sameAsTriggerQuantity |  |  |
| maxReportCells | 1 |  |  |
| reportInterval | ms1024 (1024 ms) |  |  |
| reportAmount | Infinity |  |  |
| } |  |  |  |

Table H.3.5-3a: ReportConfigEUTRA-PERIODICAL: E-UTRAN RSRP Accuracy

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-7 ReportConfigEUTRA-PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-PERIODICAL ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| purpose | reportStrongestCells |  |  |
| } |  |  |  |
| } |  |  |  |
| triggerQuantity | rsrp |  |  |
| reportQuantity | sameAsTriggerQuantity |  |  |
| maxReportCells | 2 |  |  |
| reportInterval | ms1024 (1024 ms) |  |  |
| reportAmount | Infinity |  |  |
| } |  |  |  |

ReportConfigEUTRA-PEROIDICAL: (FDD/TDD) for periodical configuration reporting of E-UTRAN RSRQ accuracy

Table H.3.5-4: ReportConfigEUTRA-PERIODICAL: E-UTRAN RSRQ Accuracy

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-7 ReportConfigEUTRA-PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-PERIODICAL ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| purpose | reportStrongestCells |  |  |
| } |  |  |  |
| } |  |  |  |
| triggerQuantity | rsrq |  |  |
| reportQuantity | sameAsTriggerQuantity |  |  |
| maxReportCells | 1 |  |  |
| reportInterval | ms1024 (1024 ms) |  |  |
| reportAmount | Infinity |  |  |
| } |  |  |  |

ReportConfigEUTRA-PEROIDICAL: (FDD/TDD) for periodical configuration reporting of E-UTRAN RSRQ accuracy

Table H.3.5-4a: ReportConfigEUTRA-PERIODICAL: E-UTRAN RSRQ Accuracy

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-7 ReportConfigEUTRA-PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-PERIODICAL ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| purpose | reportStrongestCells |  |  |
| } |  |  |  |
| } |  |  |  |
| triggerQuantity | rsrq |  |  |
| reportQuantity | sameAsTriggerQuantity |  |  |
| maxReportCells | 2 |  |  |
| reportInterval | ms1024 (1024 ms) |  |  |
| reportAmount | Infinity |  |  |
| } |  |  |  |

MeasObjectEUTRA-GENERIC(Freq): (FDD/TDD) for eICIC/feICIC (Non-MBSFN ABS)

Table H.3.5-5: MeasObjectEUTRA-GENERIC: Time Domain Measurement Resource Restriction with Non MBSFN ABS (eICIC/feICIC)

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-2 MeasObjectEUTRA-GENERIC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectEUTRA-GENERIC(Freq) ::= SEQUENCE { |  |  |  |
| neighCellConfig | '01'B (No MBSFN subframes are present in all neighbour cells) |  | FDD with E-UTRA FDD neighbour cell |
| } |  |  |  |

## H.3.6	RRC messages and information elements contents exceptions for E-UTRAN inter frequency handover and E-UTRAN intra frequency cell search

MAC-MainConfig-RBC: (FDD/TDD) to perform DRX Configuration for E-UTRAN - inter-frequency handover and E-UTRAN intra-frequency cell search with DRX_S

Table H.3.6-1: MAC-MainConfig-RBC: E-UTRAN inter frequency handover and E-UTRAN intra frequency cell search with DRX_S

| Derivation Path: 3GPP TS 36.508 [7] clause 4.8.2.1.5, Table 4.8.2.1.5-1 MAC-MainConfig-RBC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-MainConfig-RBC ::= SEQUENCE { |  |  |  |
| ul-SCH-Config SEQUENCE { |  |  |  |
| maxHARQ-Tx | n5 |  |  |
| periodicBSR-Timer | sf20 |  |  |
| retxBSR-Timer | sf1280 |  |  |
| ttiBundling | FALSE |  |  |
| } |  |  |  |
| drx-Config CHOICE { |  |  | DRX_S |
| setup SEQUENCE { |  |  |  |
| onDurationTimer | psf2 |  |  |
| drx-InactivityTimer | psf100 |  |  |
| drx-RetransmissionTimer | psf16 |  |  |
| longDRX-CycleStartOffset CHOICE { |  | sf40 typical value in real network for best-effort services. |  |
| sf40 | 0 |  |  |
| } |  |  |  |
| shortDRX | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

MAC-MainConfig-RBC: (FDD/TDD) to perform DRX Configuration for E-UTRAN - inter-frequency handover and E-UTRAN intra-frequency cell search with DRX_L

Table H.3.6-2: MAC-MainConfig-RBC: E-UTRAN inter frequency handover and E-UTRAN intra frequency cell search with DRX_L

| Derivation Path: 3GPP TS 36.508 [7] clause 4.8.2.1.5, Table 4.8.2.1.5-1 MAC-MainConfig-RBC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-MainConfig-RBC ::= SEQUENCE { |  |  |  |
| ul-SCH-Config SEQUENCE { |  |  |  |
| maxHARQ-Tx | n5 |  |  |
| periodicBSR-Timer | sf20 |  |  |
| retxBSR-Timer | sf1280 |  |  |
| ttiBundling | FALSE |  |  |
| } |  |  |  |
| drx-Config CHOICE { |  |  | DRX_L |
| Setup SEQUENCE { |  |  |  |
| onDurationTimer | psf6 |  |  |
| drx-InactivityTimer | psf1920 |  |  |
| drx-RetransmissionTimer | psf16 |  |  |
| longDRX-CycleStartOffset CHOICE { |  | sf1280 typical value in real network for best-effort services. |  |
| sf1280 | 0 |  |  |
| } |  |  |  |
| shortDRX | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

## H.3.7	RRC messages and information elements contents exceptions for E-UTRAN inter frequency and E-UTRAN inter-RAT cell search when DRX is used

MAC-MainConfig-RBC: (FDD/TDD) to perform DRX Configuration for E-UTRAN - inter-frequency and E-UTRAN inter-RAT cell search when DRX = 40 ms

Table H.3.7-1: MAC-MainConfig-RBC: E-UTRAN inter frequency cell search and E-UTRAN intra frequency cell search when DRX = 40 ms

| Derivation Path: 3GPP TS 36.508 [7] clause 4.8.2.1.5, Table 4.8.2.1.5-1 MAC-MainConfig-RBC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-MainConfig-RBC ::= SEQUENCE { |  |  |  |
| ul-SCH-Config SEQUENCE { |  |  |  |
| maxHARQ-Tx | n5 |  |  |
| periodicBSR-Timer | sf20 |  |  |
| retxBSR-Timer | sf1280 |  |  |
| ttiBundling | FALSE |  |  |
| } |  |  |  |
| drx-Config CHOICE { |  |  |  |
| Setup SEQUENCE { |  |  |  |
| onDurationTimer | psf1 |  |  |
| drx-InactivityTimer | psf1 |  |  |
| drx-RetransmissionTimer | psf1 |  |  |
| longDRX-CycleStartOffset CHOICE { |  | sf40 typical value in real network for real-time services. |  |
| sf40 | 9 | To avoid overlapping with measurement gap. |  |
| } |  |  |  |
| shortDRX | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| timeAlignmentTimerDedicated | sf500 |  |  |
| } |  |  |  |

MAC-MainConfig-RBC: (FDD/TDD) to perform DRX Configuration for E-UTRAN - inter-frequency and E-UTRAN inter-RAT cell search when DRX = 1280 ms

Table H.3.7-2: MAC-MainConfig-RBC: E-UTRAN inter frequency and E-UTRAN inter-RAT cell search when DRX = 1280 ms

| Derivation Path: 3GPP TS 36.508 [7] clause 4.8.2.1.5, Table 4.8.2.1.5-1 MAC-MainConfig-RBC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-MainConfig-RBC ::= SEQUENCE { |  |  |  |
| ul-SCH-Config SEQUENCE { |  |  |  |
| maxHARQ-Tx | n5 |  |  |
| periodicBSR-Timer | sf20 |  |  |
| retxBSR-Timer | sf1280 |  |  |
| ttiBundling | FALSE |  |  |
| } |  |  |  |
| drx-Config CHOICE { |  |  |  |
| Setup SEQUENCE { |  |  |  |
| onDurationTimer | psf1 |  |  |
| drx-InactivityTimer | psf1 |  |  |
| drx-RetransmissionTimer | psf1 |  |  |
| longDRX-CycleStartOffset CHOICE { |  | sf1280 typical value in real network for best-effort services. |  |
| sf1280 | 9 | To avoid overlapping with measurement gap. |  |
| } |  |  |  |
| shortDRX | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| timeAlignmentTimerDedicated | sf500 |  |  |
| } |  |  |  |

PhysicalConfigDedicated-DEFAULT: (FDD/TDD) to perform DRX Configuration for E-UTRAN - inter-frequency and E-UTRAN inter-RAT cell search

Table H.3.7-3: PhysicalConfigDedicated-DEFAULT: E-UTRAN inter frequency and E-UTRAN inter-RAT cell search

| Derivation Path: 3GPP TS 36.508 [7] clause 4.8.2.1.6, Table 4.8.2.1.6-1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| schedulingRequestConfig | SchedulingRequest-Config-DEFAULT |  |  |
| } |  |  |  |

## H.3.8	RRC messages and information elements contents exceptions for E-UTRAN CSI-RSRP Accuracy

MeasConfig: (FDD/TDD) to perform Measurement Configuration for E-UTRAN intra frequency CSI-RSRP

Table H.3.8-1: MeasConfig-DEFAULT: E-UTRAN intra frequency CSI-RSRP Accuracy

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-1 MeasConfig-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId | IdMeasObject-f1 | f1 is the frequency of the serving cell |  |
| measObject CHOICE { |  |  |  |
| MeasObjectEUTRA | MeasObjectEUTRA-GENERIC(f1) using condition DS_Meas |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId) )OF SEQUENCE { | 1 entry |  |  |
| reportConfigId | idReportConfig-P |  |  |
| reportConfig | ReportConfigEUTRA- PERIODICAL-CSI-RS |  |  |
| } |  |  |  |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) of SEQUENCE { | 1 entry |  |  |
| measId | 1 |  |  |
| measObjectId | IdMeasObject-f1 |  |  |
| reportConfigId | idReportConfig-P |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig-DEFAULT |  |  |
| measGapConfig | Not present |  |  |
| s-Measure | Not present |  |  |
| preRegistrationInfoHRPD | Not present |  |  |
| speedStatePars | Not present |  |  |
| } |  |  |  |

MeasConfig: (FDD/TDD) to perform Measurement Configuration for E-UTRAN inter frequency CSI-RSRP

Table H.3.8-2: MeasConfig-DEFAULT: E-UTRAN inter frequency CSI-RSRP Accuracy

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-1 MeasConfig-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f1 | f1 is the frequency of the serving cell |  |
| measObject CHOICE { |  |  |  |
| MeasObjectEUTRA | MeasObjectEUTRA-GENERIC(f1) using condition DS_Meas |  |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f2 | f2 is the frequency of the neighbouring cell (inter frequency cell) |  |
| measObject CHOICE { |  |  |  |
| MeasObjectEUTRA | MeasObjectEUTRA-GENERIC(f2) using condition DS_Meas | Inter frequency cell |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId) )OF SEQUENCE { | 1 entry |  |  |
| reportConfigId | idReportConfig-P |  |  |
| reportConfig | ReportConfigEUTRA- PERIODICAL-CSI-RS |  |  |
| } |  |  |  |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) of SEQUENCE { | 2 entries |  |  |
| measIdToAddMod[0] SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measObjectId | IdMeasObject-f1 |  |  |
| reportConfigId | idReportConfig-P |  |  |
| } |  |  |  |
| measIdToAddMod[1] SEQUENCE { |  |  |  |
| measId | 2 |  |  |
| measObjectId | IdMeasObject-f2 |  |  |
| reportConfigId | idReportConfig-P |  |  |
| } |  |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig-DEFAULT |  |  |
| measGapConfig | MeasGapConfig-GP1 |  |  |
| s-Measure | Not present |  |  |
| preRegistrationInfoHRPD | Not present |  |  |
| speedStatePars | Not present |  |  |
| } |  |  |  |

MeasConfig: (FDD/TDD) to perform Measurement Configuration for E-UTRAN Carrier Aggregation CSI-RSRP

Table H.3.8-3: MeasConfig-DEFAULT: E-UTRAN Carrier Aggregation CSI-RSRP Accuracy

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-1 MeasConfig-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f1 | f1 is the frequency of the PCell |  |
| measObject CHOICE { |  |  |  |
| MeasObjectEUTRA | MeasObjectEUTRA-GENERIC(f1) using condition DS_Meas |  |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f2 | f2 is the frequency of the SCell |  |
| measObject CHOICE { |  |  |  |
| MeasObjectEUTRA | MeasObjectEUTRA-GENERIC(f2) using condition DS_Meas |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId) )OF SEQUENCE { | 1 entry |  |  |
| reportConfigId | idReportConfig-P |  |  |
| reportConfig | ReportConfigEUTRA- PERIODICAL-CSI-RS |  |  |
| } |  |  |  |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) of SEQUENCE { | 2 entries |  |  |
| measIdToAddMod[0] SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measObjectId | IdMeasObject-f1 |  |  |
| reportConfigId | idReportConfig-P |  |  |
| } |  |  |  |
| measIdToAddMod[1] SEQUENCE { |  |  |  |
| measId | 2 |  |  |
| measObjectId | IdMeasObject-f2 |  |  |
| reportConfigId | idReportConfig-P |  |  |
| } |  |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig-DEFAULT |  |  |
| measGapConfig | Not present |  |  |
| s-Measure | Not present |  |  |
| preRegistrationInfoHRPD | Not present |  |  |
| speedStatePars | Not present |  |  |
| } |  |  |  |

ReportConfigEUTRA-PEROIDICAL-CSI-RS: (FDD/TDD) for periodical configuration reporting of E-UTRAN CSI-RSRP accuracy

Table H.3.8-4: ReportConfigEUTRA-PERIODICAL-CSI-RS: E-UTRAN CSI-RSRP Accuracy

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-12 ReportConfigEUTRA-PERIODICAL-CSI-RS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-PERIODICAL ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| purpose | reportStrongestCells |  |  |
| } |  |  |  |
| } |  |  |  |
| triggerQuantity | rsrp |  |  |
| reportQuantity | sameAsTriggerQuantity |  |  |
| maxReportCells | 2 |  |  |
| reportInterval | ms1024 |  |  |
| reportAmount | infinity |  |  |
| si-RequestForHO-r9 | Not Present |  |  |
| ue-RxTxTimeDiffPeriodical-r9 | Not Present |  |  |
| includeLocationInfo-r10 | Not Present |  |  |
| reportAddNeighMeas-r10 | Not Present |  |  |
| alternativeTimeToTrigger-r12 | Not Present |  |  |
| useT312-r12 | Not Present |  |  |
| usePSCell-r12 | Not Present |  |  |
| aN-Threshold1-v1250 | Not Present |  |  |
| a5-Threshold2-v1250 | Not Present |  |  |
| reportStrongestCSI-RSs-r12 | true |  |  |
| reportCRS-Meas-r12 | false |  |  |
| triggerQuantityCSI-RS-r12 | true |  |  |
| } |  |  |  |

QuantityConfig-DEFAULT (FDD/TDD) E-UTRAN measurement configuration for E-UTRAN CSI-RSRP accuracy

Table H.3.8-5: QuantityConfig-DEFAULT: measurement configuration for E-UTRAN CSI-RSRP accuracy

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-3A: QuantityConfig-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QuantityConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| quantityConfigEUTRA SEQUENCE { |  |  |  |
| filterCoefficientRSRP | fc0 |  |  |
| filterCoefficientRSRQ | fc0 |  |  |
| } |  |  |  |
| quantityConfigEUTRA-v1250 SEQUENCE { |  |  |  |
| filterCoefficientCSI-RSRP-r12 | fc0 |  |  |
| } |  |  |  |
| } |  |  |  |

MeasGapConfig-GP1 (FDD/TDD) E-UTRAN measurement gap configuration for E-UTRAN CSI-RSRP accuracy

Table H.3.8-6: MeasGapConfig-GP1: measurement configuration for E-UTRAN CSI-RSRP accuracy

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-1A: MeasGapConfig-GP1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig-GP1 ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| gapOffset CHOICE { |  |  |  |
| gp0 | 9 | TGRP = 40 ms |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

## H.3.9	RRC messages and information elements contents exceptions for RSSI Accuracy

MeasConfig: (FDD/TDD) to perform Measurement Configuration for E-UTRAN FS3 RSSI measurements

Table H.3.9-1: MeasConfig-DEFAULT: E-UTRAN FS3 RSSI Accuracy

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-1 MeasConfig-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId | IdMeasObject-f2 | f2 is the frequency of the serving cell |  |
| measObject CHOICE { |  |  |  |
| MeasObjectEUTRA | MeasObjectEUTRA-GENERIC(f2) |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId) )OF SEQUENCE { | 1 entry |  |  |
| reportConfigId | idReportConfig-P |  |  |
| reportConfig | ReportConfigEUTRA-PERIODICAL |  |  |
| } |  |  |  |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) of SEQUENCE { | 1 entry |  |  |
| measId | 1 |  |  |
| measObjectId | IdMeasObject-f2 |  |  |
| reportConfigId | idReportConfig-P |  |  |
| } |  |  |  |
| quantityConfig | Not present |  |  |
| measGapConfig | MeasGapConfig-GP2 |  |  |
| } |  |  |  |

ReportConfigEUTRA-PERIODICAL-RSSI: (FDD/TDD) for periodical configuration reporting of E-UTRAN FS3 RSSI accuracy

Table H.3.9-2: ReportConfigEUTRA-PERIODICAL: E-UTRAN RSSI Accuracy

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-7 ReportConfigEUTRA-PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-PERIODICAL ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| purpose | reportStrongestCells |  |  |
| } |  |  |  |
| } |  |  |  |
| reportInterval | ms120 |  |  |
| reportAmount | infinity |  |  |
| measRSSI-ReportConfig-r13 ::= SEQUENCE { | Present |  |  |
| channelOccupancyThreshold-r13 | RSSI_38 |  | ChannelOccupancy |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| ChannelOccupancy | For Channel Occupancy tests |

MeasObjectEUTRA-GENERIC (FDD/TDD) E-UTRAN measurement object configuration for FS3 RSSI measurements

Table H.3.9-3: MeasObjectEUTRA-GENERIC: measurement configuration for E-UTRAN FS3 RSSI

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-2 MeasObjectEUTRA-GENERIC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectEUTRA-GENERIC(Freq) ::= SEQUENCE { |  |  |  |
| ..measDS-Config-r12 { |  |  |  |
| ….setup ::= SEQUENCE { |  |  |  |
| dmtc-PeriodOffset-r12 | ms40-r12 |  |  |
| } |  |  |  |
| } |  |  |  |
| rmtc-Config-r13 ::= CHOICE { |  |  |  |
| ….setup ::= SEQUENCE { |  |  |  |
| rmtc-Period-r13 | ms40 |  |  |
| rmtc-SubframeOffset-r13 | 20 |  |  |
| measDuration-r13 | sym14 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

# H.4	Default RRC messages and information elements contents exceptions for Carrier Aggregation

This clause contains the default values of common RRC messages and information elements for Carrier Aggregation, other than those described in 3GPP TS 36.508 [7].

## H.4.1	RRC messages and information elements contents exceptions for E-UTRAN measurement configuration for CA

RRCConnectionReconfiguration: (FDD/TDD) to setup E-UTRAN Measurement Configuration for CA

Table H.4.1-1: Void

MeasConfig-DEFAULT: (FDD/TDD) E-UTRAN Measurement Configuration for Event Triggered Reporting for CA

Table H.4.1-2: MeasConfig-DEFAULT: E-UTRAN Measurement Configuration for CA

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-1 MeasConfig-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f1 | f1 is the frequency of the PCell (Cell 1) |  |
| measObject CHOICE { |  |  |  |
| MeasObjectEUTRA | MeasObjectEUTRA-GENERIC(f1) |  |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f2 | f2 is the frequency of the SCell on the SCC (Cell 2)and neighbouring cell on the SCC (Cell 3) |  |
| measObject CHOICE { |  |  |  |
| MeasObjectEUTRA | MeasObjectEUTRA-GENERIC(f2) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId) )OF SEQUENCE { | 1 entry |  |  |
| reportConfigId | idReportConfig-A6 |  |  |
| reportConfig | ReportConfigEUTRA-A6 |  |  |
| } |  |  |  |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) of SEQUENCE { | 1 entry |  |  |
| measId | 1 |  |  |
| measObjectId | IdMeasObject-f2 |  |  |
| reportConfigId | idReportConfig-A6 |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig-DEFAULT |  |  |
| measGapConfig | Not present |  |  |
| s-Measure | Not present |  |  |
| preRegistrationInfoHRPD | Not present |  |  |
| speedStatePars | Not present |  |  |
| } |  |  |  |

MeasObjectEUTRA-GENERIC: (FDD/TDD) E-UTRAN Measurement Configuration for Event Triggered Reporting for CA

Table H.4.1-3: MeasObjectEUTRA-GENERIC(Freq): E-UTRAN Measurement Configuration for CA with SCell measurement cycle 1280 ms)

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-2 MeasObjectEUTRA-GENERIC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectEUTRA-GENERIC(Freq) ::= SEQUENCE { |  |  |  |
| carrierFreq | Downlink EARFCN for Freq |  |  |
| allowedmeasBandwidth | The number of the resource blocks for Freq |  |  |
| presenceAntennaPort1 | FALSE |  |  |
| neighbourCellConfig | '01'B (No MBSFN subframes are present in all neighbour cells) | MBSFN doesn’t apply by default. | TDD |
|  | 10'B (The MBSFN subframe allocations of all neighbour cells are identical to or subsets of that in the serving cell) |  | FDD |
| offsetFreq | 0 (dB 0) |  |  |
| cellsToRemoveList | Not present |  |  |
| cellsToAddModList | Not present |  |  |
| excludedCellsToRemoveList | Not present |  |  |
| excludedCellsToAddModList | Not present |  |  |
| cellForWhichToReportCGI | Not present |  |  |
| measCycleSCell-r10 | sf1280 |  |  |
| measSubframePatternConfigNeigh-r10 | Not present |  |  |
| } |  |  |  |

Table H.4.1-4: Void

MeasurementReport: (FDD/TDD) E-UTRAN Measurement Report for CA and DC

Table H.4.1-5: MeasurementReport: E-UTRAN Measurement Report for CA and DC

| Derivation path: 3GPP TS 36.508 [7] clause 4.6.1 table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults ::= SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell::= SEQUENCE { |  | PCell |  |
| rsrpResult | (0..97) | Set according to specific test |  |
| rsrqResult | (0..34) | Set according to specific test |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| MeasResultEUTRA ::= SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { |  | Neighbour cell |  |
| physCellId | physCellId of Neighbour Cell |  |  |
| cgi-Info | Not present |  |  |
| measResult SEQUENCE { |  |  |  |
| rsrpResult | (0..97) | Set according to specific test |  |
| rsrqResult | (0..34) | Set according to specific test |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| locationInfo-r10 | Not present |  |  |
| measResultServFreqList-r10 SEQUENCE (SIZE (1..maxServCell-r10)) OF SEQUENCE { |  |  |  |
| servFreqId-r10 | 1 |  |  |
| measResultSCell-r10 SEQUENCE { |  | SCell |  |
| rsrpResultSCell-r10 | (0..97) | Set according to specific test |  |
| rsrqResultSCell-r10 | (0..34) | Set according to specific test |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table H.4.1-5a: MeasurementReport: E-UTRAN Measurement Report for CA and DC without Neigh Cells

| Derivation path: 3GPP TS 36.508 [7] clause 4.6.1 table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults ::= SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell::= SEQUENCE { |  | PCell |  |
| rsrpResult | (0..97) | Set according to specific test |  |
| rsrqResult | (0..34) | Set according to specific test |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| locationInfo-r10 | Not present |  |  |
| measResultServFreqList-r10 SEQUENCE (SIZE (1..maxServCell-r10)) OF SEQUENCE { |  |  |  |
| servFreqId-r10 | 1 |  |  |
| measResultSCell-r10 SEQUENCE { |  | SCell |  |
| rsrpResultSCell-r10 | (0..97) | Set according to specific test |  |
| rsrqResultSCell-r10 | (0..34) | Set according to specific test |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

ReportConfigEUTRA-A6: (FDD/TDD) E-UTRAN Measurement Report Configuration for Event A6 for CA

Table H.4.1-6: ReportConfig-A6: E-UTRAN Report config for Event A6 for CA

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6 table 4.6.6-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-A6 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA6-r10 SEQUENCE { |  |  |  |
| a6-Offset-r10 | -6 (-3 dB) | -3 is actual value in dB (-6 * 0.5 dB) | A6-Offset = -3dB |
|  | -12 (-6 dB) | -6 is actual value in dB (-12 * 0.5 dB) | A6-Offset = -6dB |
| a6-ReportOnLeave-r10 | FALSE |  |  |
| } |  |  |  |
| } |  |  |  |
| Hysteresis | 0 (0 dB) | 0 is actual value in dB (0 * 0.5 dB) |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| triggerQuantity | rsrp |  |  |
| reportQuantity | both |  |  |
| maxReportCells | 1 |  |  |
| reportInterval | ms5120 |  |  |
| reportAmount | r1 |  |  |
| } |  |  |  |

MeasurementReport: (FDD/TDD) E-UTRAN RSRP Measurement Report for CA (≥ 3DL CA)

Table H.4.1-7: MeasurementReport: E-UTRAN RSRP Measurement Report for CA (≥ 3DL CA)

| Derivation path: 3GPP TS 36.508 [7] clause 4.6.1 table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults ::= SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell::= SEQUENCE { |  | PCell |  |
| rsrpResult | (0..97) | Set according to specific test |  |
| rsrqResult | (0..34) | Set according to specific test |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| MeasResultListEUTRA ::= SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { |  | Neighbour cell |  |
| MeasResultEUTRA SEQUENCE { |  |  |  |
| physCellId | physCellId of Neighbour Cell |  |  |
| cgi-Info | Not present |  |  |
| measResult SEQUENCE { |  |  |  |
| rsrpResult | (0..97) | Set according to specific test |  |
| rsrqResult | (0..34) | Set according to specific test |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| locationInfo-r10 | Not present |  |  |
| measResultServFreqList-r10 SEQUENCE (SIZE (1..maxServCell-r10)) OF SEQUENCE { | N entries | N is E-UTRA RF Channel Number used in the test |  |
| MeasResultServFreq-r10[n] { |  |  |  |
| servFreqId-r10 | n | n denotes the index of the entry. n=1,2,…N |  |
| measResultSCell-r10 SEQUENCE { |  | SCell Cell (2*n) |  |
| rsrpResultSCell-r10 | (0..97) | Set according to specific test |  |
| rsrqResultSCell-r10 | (0..34) | Set according to specific test |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

MeasurementReport: (FDD/TDD) E-UTRAN RSRQ Measurement Report for CA (≥ 3DL CA)

Table H.4.1-8: MeasurementReport: E-UTRAN RSRQ Measurement Report for CA (≥ 3DL CA)

| Derivation path: 3GPP TS 36.508 [7] clause 4.6.1 table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults ::= SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell::= SEQUENCE { |  | PCell |  |
| rsrpResult | (0..97) | Set according to specific test |  |
| rsrqResult | (0..34) | Set according to specific test |  |
| } |  |  |  |
| measResultServFreqList-r10 SEQUENCE (SIZE (1..maxServCell-r10)) OF SEQUENCE { | N entries | N is E-UTRA RF Channel Number used in the test |  |
| MeasResultServFreq-r10[n] { |  |  |  |
| servFreqId-r10 | n |  |  |
| measResultSCell-r10 SEQUENCE { |  | SCell Cell (n+1) |  |
| rsrpResultSCell-r10 | (0..97) | Set according to specific test |  |
| rsrqResultSCell-r10 | (0..34) | Set according to specific test |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

## H.4.2	RRC messages and information elements contents exceptions for E-UTRAN RSRP and RSRQ Accuracy for CA

MeasConfig: (FDD/TDD) to perform Measurement Configuration for E-UTRAN RSRP and RSRQ accuracy for CA

Table H.4.2-1: MeasConfig-DEFAULT: E-UTRAN RSRP and RSRQ Accuracy for CA

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-1 MeasConfig-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f1 | f1 is the frequency of the PCell (Cell 1) |  |
| measObject CHOICE { |  |  |  |
| MeasObjectEUTRA | MeasObjectEUTRA-GENERIC(f1) |  |  |
| } |  |  |  |
| measObjectId | IdMeasObject-f2 | f2 is the frequency of the PCell (Cell 1) | Switch PCell/SCell scenario |
| measObject CHOICE { |  |  |  |
| MeasObjectEUTRA | MeasObjectEUTRA-GENERIC(f2) |  |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f2 | f2 is the frequency of the SCell on the SCC (Cell 2) and neighbouring cell on the SCC(Cell 3) |  |
| measObject CHOICE { |  |  |  |
| MeasObjectEUTRA | MeasObjectEUTRA-GENERIC(f2) |  |  |
| } |  |  |  |
| measObjectId | IdMeasObject-f1 | f1 is the frequency of the SCell on the SCC (Cell 2) and neighbouring cell on the SCC(Cell 3) | Switch PCell/SCell scenario |
| measObject CHOICE { |  |  |  |
| MeasObjectEUTRA | MeasObjectEUTRA-GENERIC(f1) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId) )OF SEQUENCE { | 1 entry |  |  |
| reportConfigId | idReportConfig-P |  |  |
| reportConfig | ReportConfigEUTRA- PERIODICAL |  |  |
| } |  |  |  |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) of SEQUENCE { | 1 entry |  |  |
| measIdToAddMod ::= SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measObjectId | IdMeasObject-f2 | f2 is the frequency of the SCell on the SCC (Cell 2) and neighbouring cell on the SCC (Cell 3) |  |
|  | IdMeasObject-f1 | f1 is the frequency of the SCell on the SCC (Cell 2) and neighbouring cell on the SCC (Cell 3) | Switch PCell/SCell scenario |
| reportConfigId | idReportConfig-P |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig-DEFAULT |  |  |
| measGapConfig | Not present |  |  |
| s-Measure | Not present |  |  |
| preRegistrationInfoHRPD | Not present |  |  |
| speedStatePars | Not present |  |  |
| } |  |  |  |

ReportConfigEUTRA-PEROIDICAL: (FDD/TDD) for periodical configuration reporting of E-UTRAN RSRP accuracy for CA

Table H.4.2-2: Void

ReportConfigEUTRA-PEROIDICAL: (FDD/TDD) for periodical configuration reporting of E-UTRAN RSRQ accuracy for CA

Table H.4.2-3: Void

MeasurementReport: (FDD/TDD) for periodical configuration reporting of E-UTRAN RSRP and RSRQ accuracy for CA

Table H.4.2-4: Void

MeasurementReport: (FDD/TDD) for periodical configuration reporting of E-UTRAN RSRP accuracy for CA (≥ 3DL CA)

Table H.4.2-5: MeasConfig-DEFAULT: E-UTRAN RSRP Accuracy for CA (≥ 3DL CA)

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-1 MeasConfig-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | N entries | N is E-UTRA RF Channel Number used in the test |  |
| MeasObjectToAddMod[n] SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f(n) | f(n), n=1,2,…N is the frequency of the PCell and Scells according to Annex E |  |
| measObject CHOICE { |  |  |  |
| MeasObjectEUTRA | MeasObjectEUTRA-GENERIC(f(n)) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId) )OF SEQUENCE { | 1 entry |  |  |
| reportConfigId | idReportConfig-P |  |  |
| reportConfig | ReportConfigEUTRA- PERIODICAL |  |  |
| } |  |  |  |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) of SEQUENCE { | N entries | N is E-UTRA RF Channel Number used in the test |  |
| measIdToAddMod ::= SEQUENCE { |  |  |  |
| measId | n | n denotes the index of the entry. n=1,2,…N |  |
| measObjectId | IdMeasObject-f(n) |  |  |
| reportConfigId | idReportConfig-P |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig-DEFAULT |  |  |
| measGapConfig | Not present |  |  |
| s-Measure | Not present |  |  |
| preRegistrationInfoHRPD | Not present |  |  |
| speedStatePars | Not present |  |  |
| } |  |  |  |

MeasurementReport: (FDD/TDD) for periodical configuration reporting of E-UTRAN RSRQ accuracy for CA (≥ 3DL CA)

Table H.4.2-6: MeasConfig-DEFAULT: E-UTRAN RSRQ Accuracy for CA (≥ 3DL CA)

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-1 MeasConfig-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | N entries | N is E-UTRA RF Channel Number used in the test |  |
| MeasObjectToAddMod[n] SEQUENCE { |  |  |  |
| measObjectId | IdMeasObject-f(n) | f(n) , n=1,2,…N is the frequency of the PCell and Scells according to Annex E |  |
| measObject CHOICE { |  |  |  |
| MeasObjectEUTRA | MeasObjectEUTRA-GENERIC(f(n)) |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId) )OF SEQUENCE { | 1 entry |  |  |
| reportConfigId | idReportConfig-P |  |  |
| reportConfig | ReportConfigEUTRA- PERIODICAL |  |  |
| } |  |  |  |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) of SEQUENCE { | 1 entries |  |  |
| measIdToAddMod ::= SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measObjectId | IdMeasObject-f(1) |  |  |
| reportConfigId | idReportConfig-P |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig-DEFAULT |  |  |
| measGapConfig | Not present |  |  |
| s-Measure | Not present |  |  |
| preRegistrationInfoHRPD | Not present |  |  |
| speedStatePars | Not present |  |  |
| } |  |  |  |

# H.5	Default RRC messages and information elements contents exceptions for FeICIC

This clause contains the default values of common RRC messages and information elements for FeICIC, other than those described in 3GPP TS 36.508 [7].

## H.5.1	RRC messages and information elements contents exceptions for E-UTRAN measurement configuration for FeICIC

RadioResourceConfigDedicated-SRB2-DRB(n, m): (FDD/TDD) E-UTRAN measurement configuration for NeighCells-Info-r11

Table H.5.1-1: RadioResourceConfigDedicated-SRB2-DRB(n, m): NeighCellsCRS-Info

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.3, Table 4.6.3-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| neighCellsCRS-Info-r11 ::= CHOICE { |  |  |  |
| NeighCellsCRS-Info-r11 ::= CHOICE { |  |  |  |
| setup | CRS-AssistanceInfoList-r11 |  |  |
| } |  |  |  |
| } |  |  |  |

RadioResourceConfigDedicated-SRB2-DRB(n, m): (FDD/TDD) E-UTRAN measurement configuration for CRS-AssistancecInfoList-r11

Table H.5.1-2: RadioResourceConfigDedicated-SRB2-DRB(n, m): CRS-AssistanceInfoList-r11

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.3, Table 4.6.3-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CRS-AssistancedInfoList-r11 ::= SEQUENCE (SIZE (1..maxCellReport)) OF CRS-AssistanceInfo-r11 |  |  |  |
| CRS-AssistancedInfo-r11 ::= SEQUENCE { |  |  |  |
| physCellId-r11 | (PCIcell1 - PCIcell3 )mod6 = 0 (PCIcell2 - PCIcell3 )mod6 != 0 | Cell PCIs are selected so that both conditions are met |  |
| antennaPortsCount-r11 | an1 |  |  |
| mbsfn-SubframeConfigList-r11 | MBSFN-SubframeConfigList |  |  |
| } |  |  |  |

RadioResourceConfigDedicated-SRB2-DRB(n, m): (FDD/TDD) E-UTRAN measurement configuration for MBSFN-SubframeConfigList

Table H.5.1-3: RadioResourceConfigDedicated-SRB2-DRB(n, m): MBSFN-SubframeConfigList

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.3, Table 4.6.3-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MBSFN-SubframeConfigList ::= SEQUENCE (SIZE (1..maxMBSFN-Allocations)) OF MBSFN-SubframeConfig |  |  |  |
| MBSFN-SubframeConfig:: = SEQUENCE { |  |  |  |
| radioframeAllocationPeriod | n1 |  |  |
| radioframeAllocationOffset | 0 |  |  |
| subframeAllocation CHOICE { |  |  |  |
| oneFrame | ‘000000’ | Only the CRS information of Cell 2 is provided in CRS-AssistanceInfo. It includes a single MBSFN-SubframeConfig element with subframe allocation one Frame=’000000’BIT STRING (SIZE(6)) |  |
| } |  |  |  |
| } |  |  |  |

## H.5.2	RRC messages and information elements contents exceptions for E-UTRAN RSRP and RSRQ Accuracy for FeICIC

MeasurementReport: (FDD/TDD) for periodical configuration reporting of E-UTRAN RSRP and RSRQ accuracy for FeICIC

Table H.5.2-1: MeasurementReport: E-UTRAN RSRP and RSRQ Accuracy for FeICIC

| Derivation path: 3GPP TS 36.508 [7] clause 4.6.1 Table 4.6.1-5 MeasurementReport |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults ::= SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell::= SEQUENCE { |  | PCell |  |
| rsrpResult | (0..97) | Set according to specific test |  |
| rsrqResult | (0..34) | Set according to specific test |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| MeasResultEUTRA ::= SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { |  |  |  |
| MeasResultEUTRA SEQUENCE { |  |  |  |
| physCellId | physCellId of best Cell |  |  |
| cgi-Info | Not present |  |  |
| measResult SEQUENCE { |  |  |  |
| rsrpResult | (0..97) | Set according to specific test |  |
| rsrqResult | (0..34) | Set according to specific test |  |
| } |  |  |  |
| } |  |  |  |
| MeasResultEUTRA SEQUENCE { |  |  |  |
| physCellId | physCellId of 2nd best Cell |  |  |
| cgi-Info | Not present |  |  |
| measResult SEQUENCE { |  |  |  |
| rsrpResult | (0..97) | Set according to specific test |  |
| rsrqResult | (0..34) | Set according to specific test |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| locationInfo-r10 | Not present |  |  |
| measResultServFreqList-r10 SEQUENCE (SIZE (1..maxServCell-r10)) OF SEQUENCE { |  |  |  |
| servFreqId-r10 | 1 |  |  |
| measResultSCell-r10 SEQUENCE { |  | SCell |  |
| rsrpResultSCell-r10 | (0..97) | Set according to specific test |  |
| rsrqResultSCell-r10 | (0..34) | Set according to specific test |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

# H.6	Default RRC messages and information elements contents exceptions for Proximity based services

This clause contains the default values of common RRC messages and information elements for Proximity based services, other than those described in 3GPP TS 36.508 [7].

## H.6.1	RRC messages and information elements contents exceptions for E-UTRAN measurement configuration for Proximity based services

### H.6.1.1	Reference resource pool configurations for ProSe Direct Discovery

Table H.6.1.1-1: ProSe Direct Discovery configuration for E-UTRA FDD (Configuration #1)

| Information Element |  |  |  | Value |
| --- | --- | --- | --- | --- |
| discRxPool | cp-Len |  |  | Normal |
|  | discPeriod |  |  | rf32 |
|  | numRetx |  |  | 0 |
|  | numRepetition |  |  | 1 |
|  | tf-ResourceConfig | prb-Num |  | 12 |
|  |  | prb-Start |  | 0 |
|  |  | prb-End |  | 23 |
|  |  | offsetIndicator |  | 160 |
|  |  | subframeBitmap |  | 11000000 00000000 00000000 00000000 00000000 |
|  | txParameters |  |  | not present |
|  | rxParameters |  |  | not present |
| discTxPoolCommon | cp-Len |  |  | Normal |
|  | discPeriod |  |  | rf32 |
|  | numRetx |  |  | 0 |
|  | numRepetition |  |  | 1 |
|  | tf-ResourceConfig | prb-Num |  | 2 |
|  |  | prb-Start |  | 3 |
|  |  | prb-End |  | 5 |
|  |  | offsetIndicator |  | 160 |
|  |  | subframeBitmap |  | 10000000 00000000 00000000 00000000 00000000 |
|  | txParameters | txParametersGeneral | alpha | al0 |
|  |  |  | p0 | 31 |
|  |  | ue-SelectedResourceConfig | poolSelection | random |
|  |  |  | txProbability | p100 |
|  | rxParameters |  |  | not present |
| discTxPowerInfo | discMaxTxPower |  |  | 23 |
| SL-SyncConfig | syncCP-Len |  |  | Normal |
|  | syncOffsetIndicator |  |  | 155 |
|  | slssid |  |  | 30 |
|  | txParameters | txParametersGeneral | alpha | al0 |
|  |  |  | p0 | 31 |
|  |  | syncTxThreshIC |  | 0 (-infinity) |
|  | rxParamsNCell |  |  | not present |
| discInterFreqList |  |  |  | not present |

Table H.6.1.1-2: ProSe Direct Discovery configuration for E-UTRA FDD (Configuration #2)

| Information Element |  |  |  | Value |
| --- | --- | --- | --- | --- |
| discRxPool | cp-Len |  |  | Normal |
|  | discPeriod |  |  | rf32 |
|  | numRetx |  |  | 0 |
|  | numRepetition |  |  | 1 |
|  | tf-ResourceConfig | prb-Num |  | 12 |
|  |  | prb-Start |  | 0 |
|  |  | prb-End |  | 23 |
|  |  | offsetIndicator |  | 160 |
|  |  | subframeBitmap |  | 11000000 00000000 00000000 00000000 00000000 |
|  | txParameters |  |  | not present |
|  | rxParameters | tdd-Config |  | not present |
|  |  | syncConfigIndex |  | 0 |
| discTxPoolCommon |  |  |  | not present |
| discTxPowerInfo | discMaxTxPower |  |  | 23 |
| SL-SyncConfig | syncCP-Len |  |  | Normal |
|  | syncOffsetIndicator |  |  | 140 |
|  | slssid |  |  | 30 |
|  | txParameters |  |  | not present |
|  | rxParamsNCell | physCellId |  | 1 |
|  |  | discSyncWindow |  | w1 |
| discInterFreqLis |  |  |  | not present |

Table H.6.1.1-3: ProSe Direct Discovery configuration for E-UTRA TDD Config 0 (Configuration #3)

| Information Element |  |  |  | Value |
| --- | --- | --- | --- | --- |
| discRxPool | cp-Len |  |  | Normal |
|  | discPeriod |  |  | rf32 |
|  | numRetx |  |  | 0 |
|  | numRepetition |  |  | 1 |
|  | tf-ResourceConfig | prb-Num |  | 12 |
|  |  | prb-Start |  | 0 |
|  |  | prb-End |  | 23 |
|  |  | offsetIndicator |  | 163 |
|  |  | subframeBitmap |  | 11000000 00000000 00000000 00000000 00000000 00 |
|  | txParameters |  |  | not present |
|  | rxParameters |  |  | not present |
| discTxPoolCommon | cp-Len |  |  | Normal |
|  | discPeriod |  |  | rf32 |
|  | numRetx |  |  | 0 |
|  | numRepetition |  |  | 1 |
|  | tf-ResourceConfig | prb-Num |  | 2 |
|  |  | prb-Start |  | 3 |
|  |  | prb-End |  | 5 |
|  |  | offsetIndicator |  | 163 |
|  |  | subframeBitmap |  | 10000000 00000000 00000000 00000000 00000000 00 |
|  | txParameters | txParametersGeneral | alpha | al0 |
|  |  |  | p0 | 31 |
|  |  | ue-SelectedResourceConfig | poolSelection | random |
|  |  |  | txProbability | p100 |
|  | rxParameters |  |  | not present |
| discTxPowerInfo | discMaxTxPower |  |  | 23 |
| SL-SyncConfig | syncCP-Len |  |  | Normal |
|  | syncOffsetIndicator |  |  | 158 |
|  | slssid |  |  | 30 |
|  | txParameters | txParametersGeneral | alpha | al0 |
|  |  |  | p0 | 31 |
|  |  | syncTxThreshIC |  | 0 (-infinity) |
|  | rxParamsNCell |  |  | not present |
| discInterFreqList |  |  |  | not present |

### H.6.1.2	Reference resource pool configurations for ProSe Direct Communication

Table H.6.1.2-1: ProSe Direct Communication configuration for E-UTRA FDD (Configuration #1)

| Information Element |  |  |  | Value (5MHz) | Value (10MHz) |
| --- | --- | --- | --- | --- | --- |
| commRxPool | sc-CP-Len |  |  | Normal |  |
|  | sc-Period |  |  | sf40 |  |
|  | sc-TF-ResourceConfig | prb-Num |  | 12 | 25 |
|  |  | prb-Start |  | 0 | 0 |
|  |  | prb-End |  | 23 | 49 |
|  |  | offsetIndicator |  | 0 |  |
|  |  | subframeBitmap |  | 00011000 00000000 00000000 00000000 00000000 |  |
|  | data-CP-Len |  |  | Normal |  |
|  | dataHoppingConfig | hoppingParameter |  | 0 |  |
|  |  | numSubbands |  | ns1 |  |
|  |  | rb-Offset |  | 0 |  |
|  | ue-SelectedResourceConfig | data-TF-ResourceConfig | prb-Num | 12 | 25 |
|  |  |  | prb-Start | 0 | 0 |
|  |  |  | prb-End | 23 | 49 |
|  |  |  | offsetIndicator | 0 |  |
|  |  |  | subframeBitmap | 00000000 11111111 11111111 11111111 11111111 |  |
|  |  | trpt-Subset-r12 |  | 001 |  |
|  | rxParametersNCell |  |  | not present |  |
|  | txParameters |  |  | not present |  |
| commTxPoolNormalCommon | sc-CP-Len |  |  | Normal |  |
|  | sc-Period |  |  | sf40 |  |
|  | sc-TF-ResourceConfig | prb-Num |  | 12 | 25 |
|  |  | prb-Start |  | 0 | 0 |
|  |  | prb-End |  | 24 | 49 |
|  |  | offsetIndicator |  | 0 |  |
|  |  | subframeBitmap |  | 00011000 00000000 00000000 00000000 00000000 |  |
|  | data-CP-Len |  |  | Normal |  |
|  | dataHoppingConfig | hoppingParameter |  | 0 |  |
|  |  | numSubbands |  | ns1 |  |
|  |  | rb-Offset |  | 0 |  |
|  | ue-SelectedResourceConfig | data-TF-ResourceConfig | prb-Num | 12 | 25 |
|  |  |  | prb-Start | 0 | 0 |
|  |  |  | prb-End | 23 | 49 |
|  |  |  | offsetIndicator | 0 |  |
|  |  |  | subframeBitmap | 00000000 11111111 11111111 11111111 11111111 |  |
|  |  | trpt-Subset-r12 |  | 001 |  |
|  | rxParametersNCell |  |  | not present |  |
|  | txParameters | sc-TxParameters | alpha | al0 |  |
|  |  |  | p0 | 31 |  |
|  |  | dataTxParameters | alpha | al0 |  |
|  |  |  | p0 | 31 |  |
| SL-SyncConfig | syncCP-Len |  |  | Normal |  |
|  | syncOffsetIndicator |  |  | 2 |  |
|  | slssid |  |  | 30 |  |
|  | txParameters | txParametersGeneral | alpha | al0 |  |
|  |  |  | p0 | 31 |  |
|  |  | syncTxThreshIC |  | 0 (-infinity) |  |
|  | rxParamsNCell |  |  | not present |  |

Table H.6.1.2-2: ProSe Direct Communication pre-configuration for E-UTRAN FDD for out-of-network coverage operation (Configuration #2)

| Information Element |  |  |  | Value (5MHz) | Value (10MHz) |
| --- | --- | --- | --- | --- | --- |
| preconfigSync | syncCP-Len-r12 |  |  | Normal |  |
|  | syncOffsetIndicator1 |  |  | 2 |  |
|  | syncOffsetIndicator2 |  |  | 1 |  |
|  | syncTxParameters |  |  | 31 |  |
|  | syncTxThreshOoC |  |  | 0 (-110dBm / 15kHz) |  |
|  | filterCoefficient |  |  | fc0 |  |
|  | syncRefMinHyst |  |  | dB0 |  |
|  | syncRefDiffHyst |  |  | dB0 |  |
| preconfigComm | sc-CP-Len |  |  | Normal |  |
|  | sc-Period |  |  | sf40 |  |
|  | sc-TF-ResourceConfig | prb-Num |  | 12 | 25 |
|  |  | prb-Start |  | 0 | 0 |
|  |  | prb-End |  | 23 | 49 |
|  |  | offsetIndicator |  | 0 |  |
|  |  | subframeBitmap |  | 00011000 00000000 00000000 00000000 00000000 |  |
|  | data-CP-Len |  |  | Normal |  |
|  | dataHoppingConfig | hoppingParameter |  | 0 |  |
|  |  | numSubbands |  | ns1 |  |
|  |  | rb-Offset |  | 0 |  |
|  | ue-SelectedResourceConfig | data-TF-ResourceConfig | prb-Num | 12 | 25 |
|  |  |  | prb-Start | 0 | 0 |
|  |  |  | prb-End | 23 | 49 |
|  |  |  | offsetIndicator | 0 |  |
|  |  |  | subframeBitmap | 00000000 11111111 11111111 11111111 11111111 |  |
|  |  | trpt-Subset-r12 |  | 001 |  |

# H.7	Default RRC messages and information elements contents exceptions for eMTC

This clause contains the default values of common RRC messages and information elements for eMTC, other than those described in 3GPP TS 36.508 [7].

## H.7.1	RRC messages and information elements contents exceptions for eMTC

EPDCCH-Config-r11-DEFAULT for eMTC.

Table H.7.1-1: EPDCCH-Config-r11-DEFAULT

| Derivation Path: 3GPP TS 36.508 clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPDCCH-Config-r11 ::= SEQUENCE{ |  |  |  |
| config-r11{ |  |  |  |
| setup SEQUENCE { |  |  |  |
| setConfigToAddModList-r11 SEQUENCE { | 1 entry |  |  |
| mpdcch-config-r13 CHOICE[1] { |  |  |  |
| setup SEQUENCE { |  |  |  |
| mpdcch-pdsch-HoppingConfig-r13 | on |  | CEfreqHopp |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CEfreqHopp | Used when frequency hopping is used in CE test cases |

PUCCH-Config-Common-v1310-DEFAULT for eMTC

Table H.7.1-2: PUCCH-ConfigCommon-v1310-DEFAULT

| Derivation Path: 3GPP TS 36.508 clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-ConfigCommon-v1310-DEFAULT ::= SEQUENCE { |  |  |  |
| n1PUCCH-AN-InfoList-r13 SEQUENCE (SIZE(1..maxCE-Level-r13)) OF INTEGER (0..2047) | 4 entries |  | CEmodeA |
| n1PUCCH-AN-InfoList-r13[1] | 2 |  |  |
| n1PUCCH-AN-InfoList-r13[2] | 2 |  |  |
| n1PUCCH-AN-InfoList-r13[3] | 2 |  |  |
| n1PUCCH-AN-InfoList-r13[4] | 2 |  |  |
| } |  |  |  |

Annex I (normative):
Conditions for RRM requirements applicability for operating bands

# I.1	Conditions for E-UTRAN RRC_IDLE state mobility

## I.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection

This clause defines the E-UTRAN intra-frequency RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot applicable for a corresponding operating band.

The conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection are defined in Table I.1.1-1

Table I.1.1-1: Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection

| Parameter | E-UTRA operating band groups Note 3 | Minimum RSRP Note 1 | Minimum SCH_RP Note 1 | RSRP Ês/Iot | SCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | FDD_A, TDD_A | -124 | -124 | -4 | -4 |
|  | FDD_B | -123.5 | -123.5 |  |  |
|  | FDD_C, TDD_C | -123 | -123 |  |  |
|  | FDD_D | -122.5 | -122.5 |  |  |
|  | FDD_E, TDD_E | -122 | -122 |  |  |
|  | FDD_F | -121.5 Note 2 | -121.5 Note 2 |  |  |
|  | FDD_G | -121 | -121 |  |  |
|  | FDD_H | -120.5 | -120.5 |  |  |
|  | FDD_N | -117.5 | -117.5 |  |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Clause I.4.2. NOTE 2: The condition is -122 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Clause 3.5. |  |  |  |  |  |

## I.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection

This clause defines the E-UTRAN inter-frequency RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot applicable for a corresponding operating band.

The conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection defined in Table I.1.1-1 also apply for inter-frequency E-UTRAN cells in this clause.

## I.1.3	Void

## I.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1

This clause defines the NB-IoT intra-frequency NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot applicable for a corresponding operating band. The UE category NB1 applicability of the conditions in Appendix I.1.4 is defined in clause 3.6.

The conditions for measurements of intra-frequency NB-IoT cells in normal coverage for cell re-selection are defined in Tables I.1.4-1 and I.1.4-3.

The conditions for measurements of intra-frequency NB-IoT cells in enhanced coverage for cell re-selection are defined in Tables I.1.4-2 and I.1.4-4.

Table I.1.4-1: NB-IoT intra-frequency measurements for HD-FDD in normal coverage

| Parameter | E-UTRA operating band groups Note | Minimum NRSRP | Minimum NSCH_RP | NRSRP Ês/Iot | NSCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | NFDD_G | -121 | -121 | -6 | -6 |
| NOTE: E-UTRA operating band groups are as defined in Clause 3.5. |  |  |  |  |  |

Table I.1.4-2: NB-IoT intra-frequency measurements for HD-FDD in enhanced coverage

| Parameter | E-UTRA operating band groups Note | Minimum NRSRP | Minimum NSCH_RP | NRSRP Ês/Iot | NSCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | NFDD_G | -121 | -121 | -15 | -15 |
| NOTE: E-UTRA operating band groups are as defined in Clause 3.5. |  |  |  |  |  |

Table I.1.4-3: NB-IoT intra-frequency measurements for TDD in normal coverage

| Parameter | E-UTRA operating band groups Note | Minimum NRSRP | Minimum NSCH_RP | NRS Ês/Iot | NSCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | NTDD_G | -129 | -129 | -6 | -6 |
| NOTE: E-UTRA operating band groups are as defined in Clause 3.5. |  |  |  |  |  |

Table I.1.4-4: NB-IoT intra-frequency measurements for TDD in enhanced coverage

| Parameter | E-UTRA operating band groups Note | Minimum NRSRP | Minimum NSCH_RP | NRS Ês/Iot | NSCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | NTDD_G | -138 | -138 | -15 | -15 |
| NOTE: E-UTRA operating band groups are as defined in Clause 3.5. |  |  |  |  |  |

## I.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1

This clause defines the NB-IoT inter-frequency NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot applicable for a corresponding operating band. The UE category NB1 applicability of the conditions in Appendix I.1.5 is defined in Clause 3.6.

The conditions for measurements of intra-frequency NB-IoT cells in normal coverage for cell re-selection defined in Table I.1.4-1 and I.1.4-3 also apply for inter-frequency NB-IoT cells in normal coverage in this clause.

The conditions for measurements of intra-frequency NB-IoT cells in enhanced coverage for cell re-selection defined in Table I.1.4-2 and I.1.4-4 also apply for inter-frequency NB-IoT cells in enhanced coverage in this clause.

# I.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State

## I.2.1	Conditions for E-UTRAN intra-frequency measurements

This clause defines the E-UTRAN intra-frequency SCH_RP and SCH Ês/Iot applicable for a corresponding operating band

The conditions for intra-frequency E-UTRAN measurements are defined in Table I.2.1-1

Table I.2.1-1: E-UTRAN intra-frequency measurements

| Parameter | E-UTRA operating band groups Note 3 | Minimum SCH_RP Note 1 | SCH Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm/15kHz | dB |
| Conditions | FDD_A, TDD_A | -127 | -6 |
|  | FDD_B | -126.5 |  |
|  | FDD_C, TDD_C | -126 |  |
|  | FDD_D | -125.5 |  |
|  | FDD_E, TDD_E | -125 |  |
|  | FDD_F | -124.5 Note 2 |  |
|  | FDD_G | -124 |  |
|  | FDD_H | -123.5 |  |
|  | FDD_N | -120.5 |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Clauses I.4.2. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Clause 3.5. |  |  |  |

## I.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps

This clause defines the E-UTRAN intra-frequency SCH_RP and SCH Ês/Iot applicable for a corresponding operating band

The conditions for intra-frequency E-UTRAN measurements with autonomous gap are defined in Table I.2.1-1

Table I.2.2-1: Void

## I.2.3	Conditions for E-UTRAN inter-frequency measurements

This clause defines the E-UTRAN inter-frequency SCH_RP, SCH Ês/Iot, RSRP and RSRP Ês/Iot applicable for a corresponding operating band

The conditions for inter-frequency E-UTRAN measurements with autonomous gap are defined in Table I.2.3-1

Table I.2.3-1: E-UTRAN inter-frequency measurements

| Parameter | E-UTRA operating band groups Note 3 | Minimum RSRP Note 1 | Minimum SCH_RP Note 1 | RSRP Ês/Iot | SCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | FDD_A, TDD_A | -125 | -125 | -4 | -4 |
|  | FDD_B | -124.5 | -124.5 |  |  |
|  | FDD_C, TDD_C | -124 | -124 |  |  |
|  | FDD_D | -123.5 | -123.5 |  |  |
|  | FDD_E, TDD_E | -123 | -123 |  |  |
|  | FDD_F | -122.5 Note 2 | -122.5 Note 2 |  |  |
|  | FDD_G | -122 | -122 |  |  |
|  | FDD_H | -121.5 | -121.5 |  |  |
|  | FDD_N | -118.5 | -118.5 |  |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Clauses B.4.2 and B.4.3. NOTE 2: The condition is -123 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Clause 3.5. |  |  |  |  |  |

## I.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps

This clause defines the E-UTRAN inter-frequency SCH_RP and SCH Ês/Iot applicable for a corresponding operating band.

The conditions for inter-frequency E-UTRAN measurements with autonomous gap are defined in Table I.2.4-1.

Table I.2.4-1: E-UTRAN inter-frequency measurements with autonomous gaps

| Parameter | E-UTRA operating band groups Note 3 | Minimum SCH_RP Note 1 | SCH Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm/15kHz | dB |
| Conditions | FDD_A, TDD_A | -125 | -4 |
|  | FDD_B | -124.5 |  |
|  | FDD_C, TDD_C | -124 |  |
|  | FDD_D | -123.5 |  |
|  | FDD_E, TDD_E | -123 |  |
|  | FDD_F | -122.5 Note 2 |  |
|  | FDD_G | -122 |  |
|  | FDD_H | -121.5 |  |
|  | FDD_N | -118.5 |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Clauses I.4.2 and I.4.3. NOTE 2: The condition is -123 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Clause 3.5. |  |  |  |

## I.2.5	Void

## I.2.6	Void

## I.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell

This clause defines the SCH_RP and SCH Ês/Iot for measurements in the secondary component carrier applicable for a corresponding operating band

The conditions for measurements of the secondary component carrier with deactivated SCell are defined in Table I.2.7-1.

Table I.2.7-1: Measurements of the secondary component carrier with deactivated SCell

| Parameter | E-UTRA operating band groups Note 3 | Minimum SCH_RP Note 1 | SCH Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm/15kHz | dB |
| Conditions | FDD_A, TDD_A | -127 | -6 |
|  | FDD_B | -126.5 |  |
|  | FDD_C, TDD_C | -126 |  |
|  | FDD_D | -125.5 |  |
|  | FDD_E, TDD_E | -125 |  |
|  | FDD_F | -124.5 Note 2 |  |
|  | FDD_G | -124 |  |
|  | FDD_H | -123.5 |  |
|  | FDD_N | -120.5 |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Clauses I.4.2 and I.4.3. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Clause 3.5. |  |  |  |

## I.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction

This clause defines the E-UTRAN intra-frequency SCH_RP and SCH Ês/Iot applicable for a corresponding operating band.

The conditions for intra-frequency E-UTRAN measurements under time domain measurement resource restriction are defined in Table I.2.8-1.

Table I.2.8-1 E-UTRAN intra-frequency measurements under time domain measurement resource restriction

| Parameter | E-UTRA operating band groups Note 3 | Minimum SCH_RP Note 1 | SCH Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm/15kHz | dB |
| Conditions | FDD_A, TDD_A | -127 | -7.5 |
|  | FDD_B | -126.5 |  |
|  | FDD_C, TDD_C | -126 |  |
|  | FDD_D | -125.5 |  |
|  | FDD_E, TDD_E | -125 |  |
|  | FDD_F | -124.5 Note 2 |  |
|  | FDD_G | -124 |  |
|  | FDD_H | -123.5 |  |
|  | FDD_N | -120.5 |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Clauses I.4.2 and I.4.3. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Clause 3.5. |  |  |  |

## I.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information

This clause defines the E-UTRAN intra-frequency SCH_RP and SCH Ês/Iot applicable for a corresponding operating band.

The conditions for intra-frequency E-UTRAN measurements under time domain measurement resource restriction with CRS assistance information are defined in Table I.2.9-1.

Table I.2.9-1: E-UTRAN intra-frequency measurements under time domain measurement resource restriction with CRS assistance information

| Parameter | E-UTRA operating band groups Note 3 | Minimum SCH_RP Note 1 dBm/15kHz | SCH Ês/Iot dB |
| --- | --- | --- | --- |
| Conditions | FDD_A, TDD_A | -127 | -11.07 |
|  | FDD_B | -126.5 |  |
|  | FDD_C, TDD_C | -126 |  |
|  | FDD_D | -125.5 |  |
|  | FDD_E, TDD_E | -125 |  |
|  | FDD_F | -124.5 Note 2 |  |
|  | FDD_G | -124 |  |
|  | FDD_H | -123.5 |  |
|  | FDD_N | -120.5 |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Clauses I.4.2 and I.4.3. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Clause 3.5. |  |  |  |

## I.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements

This clause defines the E-UTRAN intra-frequency SCH_RP, SCH Ês/Iot in discovery signal occasions [9], applicable for a corresponding operating band for discovery signal measurements

The conditions for E-UTRAN intra-frequency discovery signal measurements are as in Table I.2.1-1.

## I.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements

### I.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements

This clause defines the E-UTRAN inter-frequency SCH_RP, SCH Ês/Iot, RSRP, and Ês/Iot in discovery signal occasions [9], applicable for a corresponding operating band for CRS based discovery signal measurements.

The conditions for E-UTRAN inter-frequency CRS-based discovery signal measurements in discovery signal occasions are specified in Table I.2.11.1-1.

Table I.2.11.1-1: E-UTRAN inter-frequency discovery signal measurements

| Parameter | E-UTRA operating band groups Note 3 | Minimum RSRP Note 1 | Minimum SCH_RP Note 1 | RSRP Ês/Iot | SCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | FDD_A, TDD_A | -125 | -125 | -6 | -6 |
|  | FDD_B | -124.5 | -124.5 |  |  |
|  | FDD_C, TDD_C | -124 | -124 |  |  |
|  | FDD_D | -123.5 | -123.5 |  |  |
|  | FDD_E, TDD_E | -123 | -123 |  |  |
|  | FDD_F | -122.5 Note 2 | -122.5 Note 2 |  |  |
|  | FDD_G | -122 | -122 |  |  |
|  | FDD_H | -121.5 | -121.5 |  |  |
|  | FDD_N | -118.5 | -118.5 |  |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Clauses I.4.2 and I.4.3. NOTE 2: The condition is -123 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Clause 3.5. |  |  |  |  |  |

### I.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements

This clause defines the E-UTRAN inter-frequency SCH_RP, SCH Ês/Iot, CSI-RSRP, and CSI-RS Ês/Iot in discovery signal occasions [9], applicable for a corresponding operating band for CSI-RS based discovery signal measurements.

The conditions for E-UTRAN inter-frequency CRS-based discovery signal measurements in discovery signal occasions are specified in Table I.2.11.2-1.

Table I.2.11.2-1: E-UTRAN inter-frequency discovery signal measurements

| Parameter | E-UTRA operating band groups Note 3 | Minimum CSI-RSRP Note 1 | Minimum SCH_RP Note 1 | CSI-RS Ês/Iot | SCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | FDD_A, TDD_A | -125 | -125 | 0 | -6 |
|  | FDD_B | -124.5 | -124.5 |  |  |
|  | FDD_C, TDD_C | -124 | -124 |  |  |
|  | FDD_D | -123.5 | -123.5 |  |  |
|  | FDD_E, TDD_E | -123 | -123 |  |  |
|  | FDD_F | -122.5 Note 2 | -122.5 Note 2 |  |  |
|  | FDD_G | -122 | -122 |  |  |
|  | FDD_H | -121.5 | -121.5 |  |  |
|  | FDD_N | -118.5 | -118.5 |  |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Clauses I.4.2 and I.4.3. NOTE 2: The condition is -123 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Clause 3.5. |  |  |  |  |  |

## I.2.12 to I.2.14 Void

## I.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1

This clause defines the NB-IoT intra-frequency NSCH_RP and NSCH Ês/Iot applicable for a corresponding operating band. The UE category NB1 applicability of the conditions in Appendix I.2.15 is defined in 3GPP TS36.133 [4] Clause 3.6.

The conditions for intra-frequency measurements in normal coverage are defined in Table I.2.15-1 and I.2.15-3.

The conditions for intra-frequency measurements in enhanced coverage are defined in Table I.2.15-2 and I.2.15-4.

Table I.2.15-1: NB-IoT intra-frequency measurements for HD-FDD in normal coverage

| Parameter | E-UTRA operating band groups Note | Minimum NSCH_RP | NSCH Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm/15kHz | dB |
| Conditions | NFDD_G | -124 | -6 |
| NOTE: E-UTRA operating band groups are as defined in Clause 3.5. |  |  |  |

Table I.2.15-2: NB-IoT intra-frequency measurements for HD-FDD in enhanced coverage

| Parameter | E-UTRA operating band groups Note | Minimum NSCH_RP | SCH Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm/15kHz | dB |
| Conditions | NFDD_G | -124 | -15 |
| NOTE: E-UTRA operating band groups are as defined in Clause 3.5. |  |  |  |

Table I.2.15-3: NB-IoT intra-frequency measurements for TDD in normal coverrage

| Parameter | E-UTRA operating band groups Note | Minimum NSCH_RP | NSCH Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm/15kHz | dB |
| Conditions | NTDD_G | -129 | -6 |
| NOTE: E-UTRA operating band groups are as defined in Clause 3.5. |  |  |  |

Table I.2.15-4: NB-IoT intra-frequency measurements for TDD in enhanced coverrage

| Parameter | E-UTRA operating band groups Note | Minimum NSCH_RP | SCH Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm/15kHz | dB |
| Conditions | NTDD_G | -138 | -15 |
| NOTE: E-UTRA operating band groups are as defined in Clause 3.5. |  |  |  |

## I.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1

This clause defines the NB-IoT intra-frequency PRP1,2 applicable for a corresponding operating band. The UE category NB1 applicability of the conditions in Appendix I.2.16 is defined in Clause 3.1.

The conditions for intra-frequency RSTD measurements in normal coverage are defined in Table I.2.16-1 and I.2.16-3.

The conditions for intra-frequency RSTD measurements in enhanced coverage are defined in Table I.2.16-2 and I.2.16-4.

Table I.2.16-1: NB-IoT intra-frequency RSTD measurements for HD-FDD in normal coverrage

| Parameter | E-UTRA operating band groups Note | Minimum PRP1,2 Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | NFDD_G | -129 |
| NOTE: E-UTRA operating band groups are as defined in Clause 3.5 |  |  |

Table I.2.16-2: NB-IoT intra-frequency RSTD measurements for HD-FDD in enhanced coverrage

| Parameter | E-UTRA operating band groups Note | Minimum PRP1,2 Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | NFDD_G | -135 |
| NOTE: E-UTRA operating band groups are as defined in Clause 3.5 |  |  |

Table I.2.16-3: NB-IoT intra-frequency RSTD measurements for TDD in normal coverrage

| Parameter | E-UTRA operating band groups Note | Minimum PRP1,2 Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | NTDD_G | -129 |
| NOTE: E-UTRA operating band groups are as defined in Clause 3.5 |  |  |

Table I.2.16-4: NB-IoT intra-frequency RSTD measurements for TDD in enhanced coverrage

| Parameter | E-UTRA operating band groups Note | Minimum PRP1,2 Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | NTDD_G | -135 |
| NOTE: E-UTRA operating band groups are as defined in Clause 3.5 |  |  |

## I.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1

This clause defines the NB-IoT inter-frequency PRP1,2 applicable for a corresponding operating band. The UE category NB1 applicability of the conditions in Appendix I.2.17 is defined in Clause 3.1.

The conditions for intra-frequency RSTD measurements in normal coverage are defined in Table I.2.17-1 and I.2.17-3.

The conditions for intra-frequency RSTD measurements in enhanced coverage are defined in Table I.2.17-2 and I.2.17-4.

Table I.2.17-1: NB-IoT inter-frequency RSTD measurements for HD-FDD in normal coverrage

| Parameter | E-UTRA operating band groups Note | Minimum PRP1,2 Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | NFDD_G | -129 |
| NOTE: E-UTRA operating band groups are as defined in Clause 3.5 |  |  |

Table I.2.17-2: NB-IoT inter-frequency RSTD measurements for HD-FDD in enhanced coverrage

| Parameter | E-UTRA operating band groups Note | Minimum PRP1,2 Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | NFDD_G | -135 |
| NOTE: E-UTRA operating band groups are as defined in Clause 3.5 |  |  |

Table I.2.17-3: NB-IoT inter-frequency RSTD measurements for TDD in normal coverrage

| Parameter | E-UTRA operating band groups Note | Minimum PRP1,2 Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | NTDD_G | -129 |
| NOTE: E-UTRA operating band groups are as defined in Clause 3.5 |  |  |

Table I.2.17-4: NB-IoT inter-frequency RSTD measurements for TDD in enhanced coverrage

| Parameter | E-UTRA operating band groups Note | Minimum PRP1,2 Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | NTDD_G | -135 |
| NOTE: E-UTRA operating band groups are as defined in Clause 3.5 |  |  |

# I.3	Conditions for measurements performance requirements for UE

## I.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements

This clause defines the E-UTRAN intra-frequency RSRP applicable for a corresponding operating band.

The conditions for intra-frequency absolute RSRP and RSRQ accuracy requirements are defined in Table I.3.1-1.

Table I.3.1-1: Intra-frequency absolute RSRP and RSRQ Accuracy Requirements

| Parameter | E-UTRA operating band groups Note 3 | Minimum RSRP Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FDD_A, TDD_A | -127 |
|  | FDD_B | -126.5 |
|  | FDD_C, TDD_C | -126 |
|  | FDD_D | -125.5 |
|  | FDD_E, TDD_E | -125 |
|  | FDD_F | -124.5 Note 2 |
|  | FDD_G | -124 |
|  | FDD_H | -123.5 |
|  | FDD_N | -120.5 |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Clauses I.4.2 and I.4.3. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Clause 3.5. |  |  |

## I.3.2	Void

## I.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements

This clause defines the E-UTRAN inter-frequency RSRP applicable for a corresponding operating band.

The conditions for inter-frequency absolute RSRP and RSRQ accuracy requirements are defined in Table I.3.1-1.

## I.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements

This clause defines the E-UTRAN inter-frequency RSRP1,2 applicable for a corresponding operating band.

The conditions for inter-frequency relative RSRP and RSRQ accuracy requirements are defined in Table I.3.8-1.

## I.3.5 to I.3.7	Void

## I.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements

This clause defines the E-UTRAN intra-frequency RSRP1,2 applicable for a corresponding operating band.

The conditions for intra-frequency relative RSRP accuracy requirements are specified in Table I.3.8-1.

Table I.3.8-1 Intra-frequency relative RSRP accuracy requirements

| Parameter | E-UTRA operating band groups Note 3 | Minimum RSRP1,2 Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FDD_A, TDD_A | -127 |
|  | FDD_B | -126.5 |
|  | FDD_C, TDD_C | -126 |
|  | FDD_D | -125.5 |
|  | FDD_E, TDD_E | -125 |
|  | FDD_F | -124.5 Note 2 |
|  | FDD_G | -124 |
|  | FDD_H | -123.5 |
|  | FDD_N | -120.5 |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Clauses I.4.2 and I.4.3. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Clause 3.5. |  |  |

## I.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction

This clause defines the E-UTRAN intra-frequency RSRP applicable for a corresponding operating band.

The conditions for intra-frequency absolute RSRP and RSRQ accuracy requirements under time domain measurement resource restriction are as specified in Table I.3.1-1.

## I.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction

This clause defines the E-UTRAN intra-frequency RSRP1,2 applicable for a corresponding operating band.

The conditions for intra-frequency relative RSRP accuracy requirements under time domain measurement resource restriction are defined in Table I.3.8-1.

## I.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information

This clause defines the E-UTRAN intra-frequency RSRP applicable for a corresponding operating band.

The conditions for intra-frequency absolute RSRP and RSRQ accuracy requirements under time domain measurement resource restriction with CRS assistance information are as specified in Table I.3.1-1.

## I.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information

This clause defines the E-UTRAN intra-frequency RSRP1,2 applicable for a corresponding operating band.

The conditions for intra-frequency relative RSRP accuracy requirements under time domain measurement resource restriction with CRS assistance information are as specified in Table I.3.8-1.

## I.3.13	Void

## I.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements

### I.3.14.1	Conditions for Intra-frequency CRS-based measurements

This clause defines the intra-frequency RSRP in discovery signal occasions [9], applicable for a corresponding operating band for CRS based discovery signal measurements.

The conditions for intra-frequency absolute RSRP and RSRQ accuracy requirements for CRS-based discovery signal measurements in discovery signal occasions are as in Table I.3.1-1

### I.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements

This clause defines the intra-frequency CSI-RSRP in discovery signal occasions [9], applicable for a corresponding operating band for CSI-RS based discovery signal measurements.

The conditions for intra-frequency absolute CSI-RSRP accuracy requirements for CSI-RS-based discovery signal measurements in discovery signal occasions are specified in Table I.3.14.2-1

Table I.3.14.2-1: Intra-frequency Absolute CSI-RSRP Accuracy Requirements

| Parameter | E-UTRA operating band groups Note 3 | Minimum CSI-RSRP Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FDD_A, TDD_A | -127 |
|  | FDD_B | -126.5 |
|  | FDD_C, TDD_C | -126 |
|  | FDD_D | -125.5 |
|  | FDD_E, TDD_E | -125 |
|  | FDD_F | -124.5 Note 2 |
|  | FDD_G | -124 |
|  | FDD_H | -123.5 |
|  | FDD_N | -120.5 |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Clauses I.4.2 and I.4.3. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Clause 3.5. |  |  |

## I.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements

### I.3.15.1	Conditions for Intra-frequency CRS-based measurements

This clause defines the intra-frequency RSRP in discovery signal occasions [9], applicable for a corresponding operating band for CRS based discovery signal measurements.

The conditions for intra-frequency relative RSRP accuracy requirements for CRS-based discovery signal measurements in discovery signal occasions are as in Table I.3.8-1.

### I.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements

This clause defines the intra-frequency CSI-RSRP in discovery signal occasions [9], applicable for a corresponding operating band for CSI-RS based discovery signal measurements.

The conditions for intra-frequency relative CSI-RSRP accuracy requirements for CSI-RS-based discovery signal measurements in discovery signal occasions are specified in Table I.3.15.2-1.

Table I.3.15.2-1: Intra-frequency Relative CSI-RSRP Accuracy Requirements

| Parameter | E-UTRA operating band groups Note 3 | Minimum CSI-RSRP1,2 Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FDD_A, TDD_A | -127 |
|  | FDD_B | -126.5 |
|  | FDD_C, TDD_C | -126 |
|  | FDD_D | -125.5 |
|  | FDD_E, TDD_E | -125 |
|  | FDD_F | -124.5 Note 2 |
|  | FDD_G | -124 |
|  | FDD_H | -123.5 |
|  | FDD_N | -120.5 |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Clauses I.4.2 and I.4.3. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Clause 3.5. |  |  |

## I.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements

### I.3.16.1	Conditions for Inter-frequency CRS-based measurements

This clause defines the inter-frequency RSRP in discovery signal occasions [9], applicable for a corresponding operating band for CRS based discovery signal measurements.

The conditions for inter-frequency absolute RSRP and RSRQ accuracy requirements for CRS-based discovery signal measurements in discovery signal occasions are as in Table I.3.1-1.

### I.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements

This clause defines the inter-frequency CSI-RSRP in discovery signal occasions [9], applicable for a corresponding operating band for CSI-RS based discovery signal measurements.

The conditions for inter-frequency absolute CSI-RSRP accuracy requirements for CSI-RS-based discovery signal measurements in discovery signal occasions are as in Table I.3.14.2-1.

## I.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements

### I.3.17.1	Conditions for Inter-frequency CRS-based measurements

This clause defines the inter-frequency RSRP in discovery signal occasions [9], applicable for a corresponding operating band for CRS based discovery signal measurements.

The conditions for inter-frequency relative RSRP and RSRQ accuracy requirements for CRS-based discovery signal measurements in discovery signal occasions are as in Table I.3.8-1.

### I.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements

This clause defines the inter-frequency CSI-RSRP in discovery signal occasions [9], applicable for a corresponding operating band for CSI-RS based discovery signal measurements.

The conditions for inter-frequency relative CSI-RSRP accuracy requirements for CSI-RS-based discovery signal measurements in discovery signal occasions are as in Table I.3.15.2-1.

## I.3.18 to I.3.24 Void

## I.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1

This clause defines the NB-IoT intra-frequency NRSRP applicable for a corresponding operating band. The UE category NB1 applicability of the conditions in Appendix I.3.25 is defined in 3GPP TS36.133 [4] Clause 3.6.

The conditions for intra-frequency absolute NRSRP and NRSRQ accuracy requirements are defined in Table I.3.25-1.

Table I.3.25-1: NB-IoT intra-frequency absolute NRSRP and NRSRQ Accuracy Requirements

| Parameter | E-UTRA operating band groups Note | Minimum NRSRP |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | NFDD_G, NTDD_G | -139.8 |
| NOTE: E-UTRA operating band groups are as defined in Clause 3.5. |  |  |

## I.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1

This clause defines the NB-IoT inter-frequency NRSRP applicable for a corresponding operating band. The UE category NB1 applicability of the conditions in Appendix I.3.26 is defined in 3GPP TS36.133 [4] Clause 3.6.

The conditions for inter-frequency absolute NRSRP and NRSRQ accuracy requirements are defined in Table I.3.25-1.

# I.4	RRM Requirements Exceptions

## I.4.1	General

## I.4.2	Receiver sensitivity relaxation for UE supporting CA

For a UE supporting inter-band carrier aggregation configuration with uplink in one E-UTRA band, if there is a relaxation of receiver sensitivity ΔRIB,c>0 dB as defined in 3GPP TS 36.101 [2], Table 7.3.1-1A, the relevant side conditions specifying received power levels (E-UTRA RSRP, SCH_RP, PRP, CSI-RSRP, and Io) shall be increased by the amount Δ=ΔRIB,c defined for each of the downlink E-UTRA bands.

NOTE:	This side condition adjustment applies only for a UE supporting a single inter-band LTE CA band combination. For a UE supporting additional inter-band LTE CA band combinations, the ΔRIB,c for all bands supported by the UE, need to be studied TS 36.101 [2].

## I.4.3	Receiver sensitivity relaxation for UE configured with CA

### I.4.3.1	Inter-band carrier aggregation

In this clause, requirements exceptions are described for the UE configured with inter-band carrier aggregation with one uplink active in low operating band.

A relevant side condition (e.g., E-UTRA RSRP, SCH_RP, PRP, CSI-RSRP, and Io) in a requirement shall be increased by the amount Δ=L2-L1, where L1 is the reference sensitivity level specified in 3GPP TS 36.101 [2], Table 7.3.1-1, and L2 is the reference sensitivity level specified in 3GPP TS 36.101 [2], Table 7.3.1A-0a, when the following conditions are fulfilled,

-	both downlink component carriers on different bands are configured with CA and active,

-	the single uplink is active in the low operating band,

-	the exception requirements specified in 3GPP TS 36.101 [2], Table 7.3.1A-0a, apply.

If the relaxation Δ specified in this clause applies, then the relaxation specified in Clause I.4.2 should not be applied.

### I.4.3.2	Intra-band non-contiguous carrier aggregation

For a UE configured with intra-band non-contiguous carrier aggregation configuration with uplink in one E-UTRA band, if there is a relaxation of receiver sensitivity ΔRIBNC>0 as defined in 3GPP TS 36.101 [2], Table 7.3.1A-3, the relevant side conditions specifying received power levels (E-UTRA RSRP, SCH_RP, PRP, CSI-RSRP, and Io) shall be increased by the amount Δ=ΔRIBNC defined for the downlink SCC, when the following conditions are fulfilled,

-	both downlink component carriers are configured with CA and active,

-	one uplink carrier is active,

-	the exception requirements specified in 3GPP TS 36.101 [2], Table 7.3.1A-3, apply.

If the relaxation Δ specified in this clause applies, then the relaxation specified in Clause I.4.2 should not be applied.

### I.4.3.3	Inter-band carrier aggregation with operating bands without uplink band

In this clause, requirements are described for the UE configured with inter-band carrier aggregation involving one operating band without uplink band.

There is no relaxation in relevant side condition (e.g., E-UTRA RSRP, SCH_RP, PRP, CSI-RSRP, and Io) in a requirement, i.e., Δ=0, when the following conditions are fulfilled,

-	both downlink component carriers on different bands are configured with CA and active,

-	the single uplink is active in the high operating band,

-	conditions specified in 3GPP TS36.101 [2], Table 7.3.1A-0d, apply.

If Δ specified in this clause applies, then no other additional relaxation to REFSENS shall be applied.

# I.5	Void

# I.6	Conditions for V2X

## I.6.1	Void

## I.6.2	Conditions for Absolute S-RSRP Accuracy Requirements

This clause defines the S-RSRP applicable for a corresponding operating band.

The conditions for absolute S-RSRP accuracy requirements are defined in Table I.6.2-1.

Table I.6.2-1: Absolute S-RSRP Requirements

| Parameter | E-UTRA V2X operating band groups Note2 | Minimum S-RSRP Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
|  | TDD_G | -124 |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Clauses I.4.2 and I.4.3. NOTE 2: E-UTRA V2X operating band groups are as defined in Clause 3.5 for the corresponding E-UTRA operating bands. |  |  |

## I.6.3	Conditions for Relative S-RSRP Accuracy Requirements

This clause defines the S-RSRP1,2 applicable for a corresponding operating band.

The conditions for relative S-RSRP accuracy requirements are specified in Table I.6.3-1.

Table I.6.3-1: Relative S-RSRP accuracy requirements

| Parameter | E-UTRA V2X operating band groups Note 2 | Minimum S-RSRP1,2 Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
|  | TDD_G | -124 |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Clauses I.4.2 and I.4.3. NOTE 2: E-UTRA V2X operating band groups are as defined in Clause 3.5 for the corresponding E-UTRA operating bands. |  |  |

## I.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE

This clause defines the V2X SCH_RP and SCH Ês/Iot applicable for a corresponding operating band.

The conditions for selection/reselection to intra-frequency SyncRef UE are defined in Table I.6.4-1.

Table I.6.4-1: V2X synchronization measurements

| Parameter | E-UTRA V2X operating band groups Note 2 | Minimum V2X SCH_RP Note 1 | V2X SCH Ês/Iot Note 3 |
| --- | --- | --- | --- |
|  |  | dBm/15kHz | dB |
|  | TDD_G | -124 | -4 |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Clauses I.4.2 and I.4.3. NOTE 2: E-UTRA V2X operating band groups are as defined in Clause 3.5 for the corresponding E-UTRA operating bands. NOTE 3: V2X SCH Ês/Iot for a SyncRef UE is the minimum of the Ês/Iot of PSSS/PSBCH and the Ês/Iot of SSSS |  |  |  |

## I.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements

This clause defines the PSSCH-RSRP applicable for a corresponding operating band.

The conditions for absolute PSSCH-RSRP accuracy requirements are defined in Table I.6.5-1.

Table I.6.5-1: Absolute PSSCH-RSRP Requirements

| Parameter | E-UTRA V2X operating band groups Note2 | Minimum PSSCH-RSRP Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
|  | TDD_G | -124 |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Clauses I.4.2 and I.4.3. NOTE 2: E-UTRA V2X operating band groups are as defined in Clause 3.5 for the corresponding E-UTRA operating bands. |  |  |

# I.7	Conditions for sTTI and 1ms-TTI with 3 Subframe HARQ Processing

## I.7.1	Conditions for Maximum Timing Difference Between Uplink and Downlink Carriers in Carrier Aggregation

This clause defines the condition on the maximum timing difference between the earliest uplink carrier and the latest downlink carrier in carrier aggregation when a UE is configured with at least one serving cell that is configured with dl-STTI-Length-r15 or ShortProcessingTime =TRUE.

The timing difference between the earliest uplink carrier and the latest downlink carrier among all the serving cells configured to the UE is no larger than

when any of the serving cells is configured with dl-STTI-Length-r15=subslot and proc-Timeline-r15=nplus4set1,

when any of the serving cells is configured with dl-STTI-Length-r15 =subslot and proc-Timeline-r15=nplus6set2,

when any of the serving cells is configured with ShortProcessingTime =TRUE,

when any of the serving cells is configured with dl-STTI-Length-r15=slot,

when any of the serving cells is configured with dl-STTI-Length-r15=subslot and proc-Timeline-r15=nplus6set1,

when any of the serving cells is configured with dl-STTI-Length-r15=subslot and proc-Timeline-r15=nplus8set2.

The values of the parameters for i=1,… ,6 are as specified in Table I.7.2-1.

Table I.7.2-1: Maximum Subframe Timing Boundary Difference Between Earliest Uplink Carrier and Latest Downlink Carrier

| Parameter | Value |
| --- | --- |
|  | 66.67 μs (Note 1) |
|  | 166.67 μs (Note 2) |
|  | 200.00 μs (Note 3) |
|  | 309.90 μs (Note 4) |
|  | 352.08 μs (Note 5) |
|  | 452.08 μs (Note 6) |
| NOTE 1: Maximum timing advance with dl-STTI-Length-r15=subslot and proc-Timeline-r15=nplus4set1 [9] NOTE 2: Maximum timing advance with dl-STTI-Length-r15=subslot and proc-Timeline-r15=nplus6set2 [9] NOTE 3: Maximum timing advance with ShortProcessingTime = TRUE [9] NOTE 4: Maximum timing advance with dl-STTI-Length-r15=slot [9] NOTE 5: Maximum timing advance with dl-STTI-Length-r15=subslot and proc-Timeline-r15=nplus6set1 [9] NOTE 6: Maximum timing advance with dl-STTI-Length-r15=subslot and proc-Timeline-r15=nplus8set2 [9] |  |

# I.8	High level test procedure for SAN RRM tests

The following high-level steps are conducted for test cases for SAN defined in clauses 13 and 14.

-	A set of ephemeris information are pre-defined for each satellite corresponding to respective epoch times in TS 36.508.

-	The same ephemeris info will be maintained during the test (constant ephemerisInfo in all SIB31 updates), i.e. SAN RRM test cases are defined with fixed constant Delay and Doppler shift from Satellite access node to UE unless otherwise stated.

-	The range from which the constant Delay is selected is as follows:

-	For GSO  an altitude of 35,786km is considered.  The range of the one-way delay between UE and satellite is from 119.375ms to 128.79ms.

-	For NGSO an altitude of 600km and 1200km on a circular orbit are considered. The range of the one-way delay between UE and satellite is from 2ms (lowest value for LEO orbit 600km) to 6.67ms (highest for LEO orbit 1200 km).

-	The constant Doppler shift (i.e. frequency offset) is derived from the same ephemeris (i.e., orbit emulation) as the delay.

-	UE location is determined for the test. During the test, the test system shall provide the UE location to the DUT using AT commands.

-	The ephemeris and the UE location should be designed such that elevation angle relative to the UE position shall not be smaller than 30 deg during entire test time.

-	Test equipment adjusts the time and frequency of transmission according to the pre-defined  ephemeris (constant in all SIB31 updates) and UE location 
Annex J (informative):
Handling requirements and tests for different releases and UE capabilities

This annex gives guidance on how minimum requirements in different releases of 3GPP TS 36.133 [4] and different UE capabilities are handled in the specification 3GPP TS 36.521-3.

# J.1	General considerations

Same as 3GPP TS 36.521-1 [10] Annex I with the following exceptions:

-	Instead of "TS 36.101"	 use "TS 36.133"

-	Instead of "TS 36.521-1"	 use "TS 36.521-3"

-	Instead of "Annex I"	 use "Annex J"

# J.2	Concrete scenarios

## J.2.1	Tests for minimum requirements varying between releases, without introduction of new features

Same as 3GPP TS 36.521-1 [10] Annex I with the following exceptions:

-	Instead of "TS 36.101"	 use "TS 36.133"

-	Instead of "TS 36.521-1"	 use "TS 36.521-3"

-	Instead of "Annex I"	 use "Annex J"

## J.2.2	Void

Annex K (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2008-06 | RAN5#39bis | R5-082129 |  |  | R5-082129: Restructure of TS 36.521-1 and RRM proposal (Split of RRM from 36.521-1 v0.2.0 in its own specification 36.521-3 v0.1.0) |  | 0.1.0 |
| 2008-06 | RAN5#39bis | R5-082174 |  |  | Following approved TPs have been included: R5-082129: Restructure of TS 36.521-1 and RRM proposal (Split of RRM from 36.521-1 v0.2.0 in its own specification 36.521-3 v0.1.0) R5-082160: Cover for LTE E-UTRAN RRC_IDLE State Mobility text proposal R5-082161: Cover for LTE E-UTRAN RRC_CONNECTED State Mobility text proposal R5-082162: Update of 36.521-1: Introduction of HRPD and CDMA2000 in RRM test cases R5-082163: Cover for LTE UE Transmit Timing Requirements text proposal Editorial changes for Annexes | 0.1.0 | 0.2.0 |
| 2008-08 | RAN5#40 | R5-083164 |  |  | Following approved TPs have been included: R5-083051: LTE E-UTRAN RRC_IDLE State Mobility text proposal R5-083052: LTE E-UTRAN RRC_CONNECTED State Mobility text proposal R5-083053: LTE UE Transmit Timing Requirements text proposal R5-083054: LTE UE Measurement Procedures text proposal R5-083813: LTE UE Measurement Performance Requirements text proposal R5-083138: Text proposal for LTE E-UTRAN Cell Re-selection to HRPD or to cdma2000 1xRTT in 3GPP TS 36.521-3 R5-083056: RRC Connection Mobility Control text proposal R5-083164: LTE-RF 36-521-3 after RAN5#40 Editorial restructuring to clause 4 | 0.2.0 | 0.3.0 |
| 2008-10 | RAN5#40Bis | R5-084073 |  |  | Following approved TPs have been included: R5-084073: 3GPP TS 36.521-3 after RAN5#40Bis R5-084079: LTE Cell Re-Selection text proposal R5-084322: LTE FDD/FDD Handover for intra/inter frequency text proposal | 0.3.0 | 0.4.0 |
| 2008-11 | RAN5#41 | R5-085084 |  |  | Following approved TPs have been included: R5-085084 LTE-RF: 3GPP TS 36.521-3 after RAN5#41 R5-085718 LTE RRM Cell Re-Selection text proposal R5-085719 LTE RRM FDD/FDD Handover for intra/inter frequency text proposal R5-085720 E-UTRAN FDD intra-frequency measurements text proposal R5-085740 RSRQ Accuracy Measurement Performance Requirements text proposal R5-085722 Text Proposal for Cell Configuration mapping annex in 36.521-3 Editor’s cleanup | 0.4.0 | 0.5.0 |
| 2009-01 | RAN5#41Bis | R5-086067 |  |  | Following approved TPs have been included: R5-086067 LTE-RF: 3GPP TS 36.521-3 after RAN5#41Bis R5-086149 References to connection diagrams R5-086418 LTE RRM Cell Re-Selection text proposal R5-086095 Cell configuration reference correction for RRM tests in 36.521-3 clause 3A.3 R5-086419 LTE RRM FDD/FDD Handover for intra/inter frequency text proposal R5-086420 E-UTRAN FDD intra-frequency measurements text proposal R5-086431 RSRQ Accuracy Measurement Performance Requirements text proposal R5-086082 LTE UE Transmit Timing Requirements text proposal R5-086422 Text proposal for RSRP measurement accuracy test cases R5-086432 E-UTRAN FDD- FDD Inter-Frequency Measurements text proposal R5-086142 Measurement Reference Channels and OCNG for RRM testing R5-086150 Statistical testing in RRM tests Editor’s cleanup | 0.5.0 | 0.6.0 |
| 2009-03 | RAN5#42 | R5-090191 |  |  | Following approved TPs have been included: R5-091026 TDD Intra frequency RSRQ Accuracy R5-091085 TDD Inter frequency RSRQ Accuracy R5-091035 LTE RRM FDD/FDD Handover for intra/inter frequency text proposal R5-091047 E-UTRAN FDD intra-frequency measurements text proposal R5-091029 RSTQ Accuracy Measurement Performance Requirements text proposal R5-091041 LTE RRM E-UTRA FDD to GSM Cell Re-Selection text proposal R5-091040 LTE RRM E-UTRA FDD to GSM Handover text proposal R5-090182 LTE UE Measurement Procedures Structure text proposal R5-091048 LTE RRM E-UTRA FDD to UTRA FDD Cell Search text proposal R5-090184 LTE UE inter-RAT Handover Structure text proposal R5-091039 LTE RRM E-UTRA FDD to UTRA FDD Handover text proposal R5-091053 LTE UE Transmit Timing Requirements text proposal R5-090191 LTE-RF: 3GPP TS 36.521-3 after RAN5#42 R5-091091 Intra-frequency cell search TDD R5-091088 Intra-frequency Absolute RSRP measurement accuracy TDD R5-091090 Intra-frequency Relative RSRP measurement accuracy TDD R5-091089 Inter-frequency RSRP absolute accuracy TDD R5-091087 Inter-frequency RSRP relative accuracy TDD R5-091028 Text Proposal for RSRP Measurement Accuracy test cases R5-091076 Text Proposal for Annex C of TS 36.521-3 R5-091051 TP of E-UTRAN TDD & GSM cell re-selection test case R5-091043 TP of E-UTRAN TDD & TDD inter frequency cell re-selection test case R5-091036 TP of E-UTRAN TDD & TDD inter frequency handover test case R5-091044 TP of E-UTRAN TDD - TDD intra frequency cell re-selection test case R5-091038 TP of E-UTRAN TDD & TDD intra frequency handover test case R5-091045 TP of E-UTRAN TDD & UTRAN TDD cell re-selection test case R5-091049 E-UTRAN FDD- FDD Inter-Frequency Measurements text proposal R5-091050 E-UTRAN TDD- TDD Inter-Frequency Measurements text proposal R5-091052 LTE-RF: Update to 36.521-3 Annex E Cell Configuration mapping R5-091064 Correction to frequencies to be tested in RRM test cases R5-091042 LTE RRM Cell Re-Selection text proposal Editor’s cleanup | 0.6.0 | 1.0.0 |
| 2009-03 | RAN5#42Bis |  |  |  | R5-091263 LTE-RRM Cell Re-Selection text proposal R5-091922 LTE-RRM E-UTRA FDD to GSM Cell Re-Selection text proposal R5-091923 LTE-RRM E-UTRA FDD - UTRA TDD Cell Re-Selection text proposal R5-091924 TP of E-UTRA TDD - GSM cell reselection R5-091945 TP of E-UTRA TDD-UTRAN TDD cell re-selection : UTRA is of higher priority R5-091926 TP of E-UTRA TDD - UTRA TDD cell reselection: UTRA is of lower priority test case R5-091264 LTE-RRM FDD/FDD Handover for intra/inter frequency text proposal R5-091931 LTE-RRM E-UTRA FDD to GSM Handover text proposal R5-091928 LTE-RRM E-UTRA FDD to UTRA FDD Handover text proposal R5-091946 LTE-RRM: E-UTRA TDD to UTRA FDD Handover text proposal R5-091947 LTE-RRM: Handover test proposal R5-091930 TP of E-UTRA TDD to UTRA TDD handover test case R5-091265 LTE-RRM E-UTRAN FDD intra-frequency measurements text proposal R5-091266 LTE-RRM RSRQ Accuracy Measurement Performance Requirements text proposal R5-091932 LTE-RRM E-UTRA FDD to UTRA FDD Cell Search (fading) text proposal R5-091933 LTE-RRM E-UTRAN FDD & UE Transmit Timing Accuracy text proposal R5-091276 LTE-RRM E-UTRAN FDD & UE Timing Advance Adjustment Accuracy text proposal R5-091934 LTE-RRM: E-UTRA TDD to UTRA FDD Cell Search (fading) text proposal R5-091935 LTE-RRM E-UTRAN TDD & UE Transmit Timing Accuracy text proposal R5-091936 LTE-RRM E-UTRAN TDD & UE Timing Advance Adjustment Accuracy text proposal R5-091937 LTE-RRM E-UTRA FDD to UTRA TDD Cell Search (fading) text proposal R5-091381 EUTRAN TDD to UTRAN TDD cell search (fading) R5-091386 LTE RRM TDD Inter Frequency RSRP Accuracy text proposal R5-091398 Text Proposal for RSRP Measurement Accuracy test cases R5-091948 LTE-RRM: Measurements test proposal R5-091431 RRM-EUTRAN FDD RLM test for out-of-sync R5-091434 RRM-EUTRAN TDD RLM test for out-of-sync R5-091435 RRM-EUTRAN FDD RLM test for In-sync R5-091436 RRM-EUTRAN TDD RLM test for In-sync R5-091468 RRM E-UTRAN FDD-FDD Inter-frequency Measurements R5-091469 RRM E-UTRAN TDD-TDD Inter-frequency Measurements R5-091939 LTE-RRM cell configuration mapping updates R5-091407 Update of statistical requirements to 36.521-3 Editor’s cleanup | 1.0.0 | 1.1.0 |
| 2009-05 | RAN5#43 | R5-092156 |  |  | R5-092156 LTE-RF: 3GPP TS 36.521-3 after RAN5#43 R5-092066 E-UTRAN FDD- FDD Inter-Frequency Measurements text proposal R5-092617 RRM E-UTRAN TDD-TDD Inter-frequency Measurement R5-092068 RRM-EUTRAN FDD RLM test for out-of-sync and in-synch R5-092069 RRM-EUTRAN TDD RLM test for out-of-sync and in-synch R5-092071 Reference measurement Channels for Radio Link Monitoring Tests with 2 Antennas R5-092127 Update of statistical requirements to 36.521-3 R5-092630 LTE RRM: 12 RX antenna R5-092618 Text Proposal for E-UTRAN FDD-UTRAN FDD cell re-selection test cases R5-092651 Text Proposal for E-UTRAN FDD - GSM Measurements test case R5-092620 LTE-RRM E-UTRA TDD to GSM event triggered reporting in AWGN text proposal R5-092360 LTE RRM TDD Inter Frequency RSRP Accuracy text proposal R5-092621 LTE-RRM Default Message Contents for support of RRM text proposal R5-092384 LTE-RRM Update of Message Contents for E-UTRAN FDD RRM tests to align with support of RRM text proposal R5-092622 LTE-RRM E-UTRAN FDD Cell Re-Selection text proposal R5-092386 LTE-RRM FDD Inter Frequency RSRQ Accuracy Measurement Performance Requirements text proposal R5-092387 LTE-RRM E-UTRA FDD to GSM Cell Re-Selection text proposal R5-092623 LTE-RRM E-UTRAN FDD intra frequency cell search (fading) in long DRX text proposal R5-092624 LTE-RRM E-UTRAN FDD intra frequency cell search (fading) in short DRX text proposal R5-092390 LTE-RRM E-UTRAN FDD - UE Transmit Timing Accuracy text proposal R5-092626 LTE-RRM E-UTRAN FDD random access: contention based scenario text proposal R5-092627 LTE-RRM E-UTRAN FDD random access: non-contention based scenario text proposal R5-092628 LTE-RRM E-UTRA FDD to HRPD Handover text proposal R5-092629 LTE-RRM E-UTRA FDD to cdma2000 1xRTT Handover text proposal R5-092443 Addition of band 18 and 19 to LTE RRM test cases Editor’s cleanup | 1.1.0 | 2.0.0 |
| 2009-05 | RAN#44 | - | - |  | Updated to v8.0.0 after RAN#44 with no technical change. | 2.0.0 | 8.0.0 |
| 2009-06 | - | - | - |  | Editorial clean up | 8.0.0 | 8.0.1 |
| 2009-09 | RAN#45 | R5-094036 | 0001 | - | Correction CR to 36.521-3: Update of Requirements for E-UTRAN (FDD) cell re-selection tests | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094037 | 0002 | - | Correction CR to 36.521-3: Update of Requirements for E-UTRAN FDD - FDD Inter Frequency Handover test | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094038 | 0003 | - | Correction CR to 36.521-3: Update of Requirements for E-UTRAN FDD - FDD Intra Frequency Cell Search test | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094039 | 0004 | - | Correction CR to 36.521-3: Update of Requirements for E-UTRAN FDD - UE transmit timing accuracy test | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094040 | 0005 | - | Correction CR to 36.521-3: Update of Requirements for E-UTRAN FDD - GSM cell re-selection test | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094041 | 0006 | - | Correction CR to 36.521-3: Update of Requirements conditions for E-UTRAN FDD - UE timing advance adjustment accuracy test | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094042 | 0007 | - | Correction CR to 36.521-3: Update of Requirements for E-UTRAN FDD - GSM Handover test | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094043 | 0008 | - | Correction CR to 36.521-3: Update of Requirements for E-UTRAN FDD - UTRAN FDD Handover test | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094045 | 0009 | - | Correction CR to 36.521-3: Update of Requirements for E-UTRAN FDD - GSM Cell Search test | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094047 | 0010 | - | Correction CR to 36.521-3: Update of Requirements for E-UTRAN FDD - Contention Based Random Access test | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094048 | 0011 | - | Correction CR to 36.521-3: Update of Requirements for E-UTRAN FDD - Non-Contention Based Random Access test | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094049 | 0012 | - | Addition of test scenario CR to 36.521-3: E-UTRAN FDD-FDD Inter-frequency cell search when DRX is used under fading propagation conditions | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094050 | 0013 | - | Correction CR to 36.521-3: Update of E-UTRAN FDD-FDD Intra-frequency cell search when DRX is used under fading propagation conditions | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094051 | 0014 | - | Correction CR to 36.521-3: Update of Annex H Default Message Contents for support of RRM | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094217 | 0015 | - | Update for E-UTRA FDD - UTRA TDD cell reselection | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094218 | 0016 | - | Test proposal for E-UTRA FDD - UTRA TDD HO | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094219 | 0017 | - | Test proposal for E-UTRA TDD random access: contention based scenario | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094220 | 0018 | - | Test proposal for E-UTRA TDD random access: non-contention based scenario | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094221 | 0019 | - | Update for TDD Intra-frequency RSRQ measurement accuracy | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094222 | 0020 | - | Update for TDD Inter-frequency RSRQ measurement accuracy | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094223 | 0021 | - | Update for E-UTRAN TDD Transmit timing accuracy | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094225 | 0022 | - | Update for E-UTRA FDD - UTRA TDD cell search(fading) | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094253 | 0023 | - | CR to 36.521-3: Addition of E-UTRAN FDD Intra-frequency RRC Re-establishment | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094254 | 0024 | - | CR to 36.521-3: Addition of E-UTRAN FDD Inter-frequency RRC Re-establishment | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094285 | 0025 | - | LTE-RRM: Introduction of Common Exception messages table for E-UTRAN TDD-UTRAN FDD handover and E-UTRAN TDD-UTRAN FDD measurements | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094358 | 0026 | - | Correction to RSRP measurement accuracy test cases | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094442 | 0027 | - | CR to 36.521-3: Update of E-UTRA FDD to HRPD Handover and E-UTRA FDD to cdma2000 1xRTT Handover test cases | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094709 | 0028 | - | LTE RRM: Correction to test cases 4.4.1 and 4.4.2 | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094713 | 0029 | - | Resubmission - Update to E-UTRAN to HRPD Cell Re-Selection (HRPD is of lower priority) test case | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094720 | 0030 | - | Resubmission - Update to E-UTRAN to CDMA2000 1xRTT Cell Re-Selection (CDMA2000 1xRTT is of lower priority) test case | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094743 | 0031 | - | RRM TCs in test mode | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094927 | 0032 | - | Correction CR to 36.521-3: Update of inter-frequency E-UTRAN TDD-TDD cell re-selection 4.2.6 | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094928 | 0033 | - | Correction CR to 36.521-3: Update of E-UTRAN TDD - UTRAN TDD cell re-selection 4.3.4 | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094929 | 0034 | - | Correction CR to 36.521-3: Update of Requirements for E-UTRAN FDD - UTRAN FDD cell re-selection test | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094930 | 0035 | - | LTE-RRM: Addition of common messages to Annex H | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094931 | 0036 | - | Test Proposal for E-UTRAN TDD Intra-frequency RRC Re-establishment | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094932 | 0037 | - | Test Proposal for E-UTRAN TDD Inter-frequency RRC Re-establishment | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094933 | 0038 | - | Update for E-UTRAN TDD Timing advanced adjustment accuracy | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094934 | 0039 | - | Correction CR to 36.521-3: Update of Requirements for E-UTRAN FDD - UTRAN FDD Cell Search test | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094935 | 0040 | - | E-UTRA TDD - TDD Intra frequency cell search with DRX | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094936 | 0041 | - | TDD - TDD RSRP measurement | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094937 | 0042 | - | Update 8.10.1 E-UTRAN TDD-GSM event triggered reporting in AWGN | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094938 | 0043 | - | Add new tc 8.10.2 EUTRAN TDD-GSM cell search with DRX | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094939 | 0044 | - | Add new tc 8.7.2 EUTRAN TDD - UTRAN TDD cell search with DRX | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094940 | 0045 | - | E-UTRA TDD - TDD Inter frequency cell search with DRX | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094942 | 0046 | - | Update to Annex E Cell Configuration Mapping | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094967 | 0047 | - | RRM Radio Link Monitoring FDD update | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094968 | 0048 | - | RRM Radio Link Monitoring TDD update | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094969 | 0050 | - | RRM: E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094970 | 0051 | - | CR to 36.521-3:Message updates for RSRP and RSRQ Accuracy measurement | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094971 | 0052 | - | RRM OCNG and RMC update | 8.0.1 | 8.1.0 |
| 2009-09 | RAN#45 | R5-094972 | 0053 | - | RRM: Update of Annex E for SON | 8.0.1 | 8.1.0 |
| 2009-12 | RAN#46 | R5-095492 | 0054 | - | Removal of test state 4 in RRM test cases | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-095493 | 0055 | - | CR to 36.521-3 Annexes of E-UTRAN cell reselection test cases | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-095499 | 0056 | - | CR for E-UTRAN FDD - UTRAN TDD handover | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-095501 | 0057 | - | CR for E-UTRAN TDD - UE Transmit Timing Accuracy | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-095503 | 0058 | - | CR for E-UTRAN FDD - UTRAN TDD event triggered reporting under fading propagation conditions | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-095504 | 0059 | - | Correction to TDD RSRP and RSRQ measurement requirements | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-095527 | 0060 | - | Correction CR to 36.521-3: E-UTRAN FDD - FDD cell re-selection intra frequency case and inter frequency case conformance minimum requirements updates | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-095528 | 0061 | - | Correction CR to 36.521-3: E-UTRAN FDD - UTRAN FDD cell re-selection: UTRA FDD is of higher priority and UTRA FDD is of lower priority conformance minimum requirements | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-095529 | 0062 | - | Correction CR to 36.521-3: E-UTRAN FDD - FDD Inter Frequency event triggered reporting under fading propagation conditions in asynchronous cells case | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-095530 | 0063 | - | Correction CR to 36.521-3: E-UTRAN FDD - FDD Inter Frequency event triggered reporting when DRX is used under fading propagation conditions | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-095531 | 0064 | - | Correction CR to 36.521-3: E-UTRAN FDD - FDD Intra Frequency event triggered reporting under fading propagation conditions | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-095537 | 0065 | - | Correction CR to 36.521-3: E-UTRAN FDD - UE Transmit Timing Accuracy case | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-095538 | 0066 | - | Correction CR to 36.521-3: E-UTRAN FDD - FDD inter frequency event triggered reporting when DRX is used | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-095557 | 0067 | - | Correction CR to 36.521-3: General RRM Updates | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-095572 | 0068 | - | Update TC 8.7.2 E-UTRAN TDD - UTRAN TDD cell search when DRX is used under fading propagation conditions | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-095573 | 0069 | - | Update TC 8.10.2 E-UTRAN TDD - GSM event triggered reporting when DRX is used in AWGN | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-095576 | 0070 | - | Update TC 8.2.1 E-UTRAN TDD - TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-095591 | 0071 | - | update of Annex H.2.3 in 36.521-3 | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-095741 | 0072 | - | CR to the inconsistent expression in UE Measurements Procedures | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-095917 | 0073 | - | Update: Radio Link Monitoring test cases: no DRX | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096145 | 0074 | - | Correction CR to 36.521-3: E-UTRAN FDD - FDD Inter Frequency Absolute RSRP Accuracy case | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096243 | 0075 | - | Update to RRM TC:E-UTRAN FDD - UTRAN TDD cell re-selection | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096244 | 0104 | 1 | Addition of new TC to 36.521-3: E-UTRAN TDD - UTRAN FDD cell re-selection test | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096246 | 0105 | - | Modification of clause 4.2.2 in 36.521-3 | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096247 | 0106 | - | Modification of clause 4.2.2 in 36.521-3 | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096255 | 0076 | - | CR to the RA response window's name in Random Access conformance requirements | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096257 | 0077 | - | Addition of test scenario CR to 36.521-3: E-UTRAN FDD - UTRAN FDD cell re-selection in fading propagation conditions: UTRA FDD is of lower priority | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096258 | 0078 | 1 | Addition of new TC to 36.521-3:E-UTRAN TDD - GSM Handover: Unknown Target Cell | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096263 | 0079 | 1 | Add new TC 5.1.6 E-UTRAN TDD - TDD inter frequency handover: unknown target cell | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096265 | 0080 | - | CR to 36.521-3: Update of E-UTRA FDD to HRPD Handover and E-UTRA FDD to cdma2000 1xRTT Handover test cases | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096267 | 0081 | - | Addition of test scenario CR to 36.521-3: E-UTRAN FDD - GSM Cell Search when DRX is used | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096268 | 0082 | - | Addition of test scenario CR to 36.521-3: E-UTRAN FDD - UTRAN FDD Cell Search when DRX is used | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096269 | 0083 | - | RRM: Update of test case 8.4.1 TDD inter-frequency event triggered reporting | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096271 | 0084 | - | LTE-RF: Update to Annex E Cell Configuration Mapping | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096272 | 0085 | - | Correction CR to 36.521-3: Addition of message contents exceptions for E-UTRAN inter frequency and inter-RAT Cell Search for when DRX is used | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096273 | 0086 | - | Correction CR to 36.521-3: E-UTRAN FDD - UTRA FDD Handover case | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096274 | 0087 | - | CR to 36.521-3: Update to FDD Intra-frequency RRC Re-establishment test case | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096275 | 0088 | - | CR to 36.521-3: Update to FDD Inter-frequency RRC Re-establishment test case | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096276 | 0107 | - | Test Case of E-UTRAN TDD to GSM Handover | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096296 | 0089 | - | Update TC 8.7.1 E-UTRAN TDD - UTRAN TDD event triggered reporting under fading propagation conditions | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096302 | 0090 | - | Correction CR to 36.521-3: E-UTRAN FDD - GSM event triggered reporting in AWGN case | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096303 | 0091 | - | Addition of new TC to 36.521-3: E-UTRAN TDD to E-UTRAN TDD and UTRA TDD cell search test case | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096310 | 0092 | - | Addition of test scenario CR to 36.521-3: E-UTRAN FDD - UTRAN FDD Handover: Unknown Target Cell | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096324 | 0093 | - | Addition of new TC to 36.521-3 E-UTRAN TDD - UTRAN TDD HO test: unknown target cell | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096325 | 0094 | - | Addition of test scenario CR to 36.521-3: E-UTRAN FDD - FDD inter frequency Handover test cases: Unknown Target Cell | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096326 | 0095 | - | Addition of test scenario CR to 36.521-3: E-UTRAN FDD - GSM Handover: Unknown Target Cell | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096327 | 0096 | - | Addition of new TC to 36.521-3: E-UTRAN TDD - E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions test case | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096328 | 0097 | - | E-UTRAN FDD Radio Link Monitoring test for Out-of-sync in DRX | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096329 | 0098 | - | E-UTRAN FDD Radio Link Monitoring test for in-sync in DRX | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096330 | 0099 | - | E-UTRAN TDD Radio Link Monitoring test for Out-of-sync in DRX | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096331 | 0100 | - | E-UTRAN TDD Radio Link Monitoring test for in-sync in DRX | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096332 | 0101 | - | RRM: Update of test case 8.3.1 FDD inter-frequency event triggered reporting | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096337 | 0102 | - | Correction CR to 36.521-3: E-UTRAN FDD - FDD Handover intra frequency and inter frequency case | 8.1.0 | 8.2.0 |
| 2009-12 | RAN#46 | R5-096340 | 0103 | - | Introduction of uncertainties for RRM test cases 4.2.1 and 4.2.2 | 8.1.0 | 8.2.0 |
| 2010-03 | RAN#47 | R5-100130 | 0108 | - | Test Tolerances and alignment with 36.133 for cell re-selection intra frequency cases | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100132 | 0109 | - | Uncertainties and Test Tolerances for inter frequency cell re-selection | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100135 | 0110 | - | Clarification of Extreme conditions for RSRP test cases | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100362 | 0113 | - | CR about the Cell Search Requirements for LTE FDD-FDD/ TDD-TDD Handover to Unknown Target Cell | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100365 | 0114 | - | CR on updating the handover delay requirements for E-UTRAN TDD - TDD both intra-frequency and inter-frequency handovers | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100367 | 0115 | - | CR to correct the test requirements of reselection from E-UTRAN FDD/TDD to UTRAN TDD | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100394 | 0116 | - | Correction of Annex H about measurement performance messages | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100401 | 0117 | - | RRM Inter frequency cell search updates, TC 8.3.1 and 8.4.1 | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100438 | 0118 | - | Update TC 8.7.1, 8.9.1 and 8.11.4 | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100460 | 0119 | - | Misc update on 521-3 | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100486 | 0120 | - | CR to 36.521-3: Addition of E-UTRA FDD to HRPD Handover: Unknown Target Cell and E-UTRA FDD to cdma2000 1xRTT Handover: Unknown Target Cell test cases | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100519 | 0121 | - | Correction to RSRP Accuracy test cases | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100546 | 0122 | - | CR to 36.521-3: Update to E-UTRAN FDD RRC Re-establishment test cases | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100562 | 0123 | - | CR to 36.521-3: Update LTE RRM test cases with test requirements for extended LTE1500 | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100714 | 0124 | - | Addition of missing Es/Noc parameters in RRM test cases | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100715 | 0125 | - | Correction to GSM measurement configuration in Annex H.3.1 | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100716 | 0126 | - | Update on Annex C for 36.521-3 | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100849 | 0127 | - | Text on exclusion of extra delay due to RRC retransmission | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100850 | 0128 | - | Correction to test iteration procedure in cell re-selection TCs | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100852 | 0129 | - | DL Mac Padding for RRM TCs | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100853 | 0130 | - | Update TC 5.1.6 E-UTRAN TDD-TDD inter frequency handover unknown target cell | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100854 | 0131 | - | New RRM test case, 8.7.3 E-UTRAN TDD SON ANR | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100859 | 0132 | - | Update TC 8.2.2 E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100860 | 0133 | - | Update TC 8.2.1 E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100861 | 0134 | - | Update TC 8.4.2 E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100862 | 0135 | - | Misc update on test applicability | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100865 | 0136 | - | CR about corrections of PDSCH Reference Measurement Channels | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100866 | 0137 | - | CR about OFDMA Channel Noise Generator (OCNG) | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100873 | 0138 | - | CR to 36.521-3 Rel-8 Introduction of E-UTRAN FDD - FDD Intra Frequency Cell Search with DRX when L3 filtering is used | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100890 | 0139 | - | Update to RRM TC: TDD Intra frequency RSRP Accuracy | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100896 | 0140 | - | Clarification on Time offset between cells | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100897 | 0141 | - | Update to RRM TC:E-UTRAN TDD-TDD cell re-selection | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100898 | 0142 | - | Update to RRM TC: TDD Inter frequency RSRP Accuracy | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100900 | 0143 | - | Uncertainties and Test Tolerances for FDD Intra Frequency Absolute RSRP Accuracy | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | R5-100901 | 0144 | - | RRM TTIdcch and cell timing change, update of chapter 8 | 8.2.0 | 8.3.0 |
| 2010-03 | RAN#47 | - | - | - | Moved to v9.0.0 with no change | 8.3.0 | 9.0.0 |
| 2010-06 | RAN#48 | R5-103105 | 0145 | - | CR to 36.521-3: Update RSRP test cases with band 11 and 21 | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103116 | 0146 | - | Correction of CR conflict for Intra Frequency TDD reselection test | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103117 | 0147 | - | Reference to TR 36.903 in 3GPP TS 36.521-3 | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103312 | 0149 | - | Connection diagram for test 8.11.2 (3 cells) | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103315 | 0150 | - | Correction to connection diagram reference for test 8.10.1 and 8.10.2 | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103330 | 0151 | - | update on test applicability | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103358 | 0152 | - | Annex E update | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103496 | 0153 | - | LTE-RRM: Update of test procedure for measurement performance test cases | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103526 | 0154 | - | CR 36.521-3 on corrections to requirements in Idle Mode | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103528 | 0155 | - | CR 36.521-3 on correction to InterRAT handover minimum requirements | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103531 | 0156 | - | CR 36.521-3 on correction to measurement requirements | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103532 | 0157 | - | CR 36.521-3 on correction to E-UTRA inter frequency cell search requirements | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103534 | 0158 | - | CR 36.521-3 on correction to UE transmit timing minimum and test requirements | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103541 | 0159 | - | Addition of test tolerances and system uncertainties for E-UTRAN FDD-FDD HO inter-frequency case | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103546 | 0160 | - | Addition of test tolerances and system uncertainties for E-UTRAN FDD-FDD intra frequency cell search under fading in asynchronous cells | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103547 | 0161 | - | Addition of test tolerances and system uncertainties for E-UTRAN FDD-FDD intra frequency cell search under fading in synchronous cells | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103548 | 0162 | - | Addition of test tolerances and system uncertainties for E-UTRAN TDD-TDD intra frequency cell search under fading in synchronous cells | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103607 | 0163 | - | Correction to step of physical cell identity change in 4.2.3 | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103608 | 0164 | - | Correction of test mode reference to 36.508 | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103611 | 0165 | - | Correction to the references of exceptional message | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103612 | 0166 | - | Correction to b2-Threshold1 in the exceptional message | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103613 | 0194 | - | Correction to Radio Resource Configuration in UE transmit timing and UE timing advance TCs | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103614 | 0195 | - | Correction to Gap Pattern Id in the exceptional message | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103615 | 0196 | - | Correction to Measure object and ID in the exceptional messages | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103658 | 0197 | - | Iteration in cell reselection tests | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103709 | 0167 | - | Connection diagram reference for intra-freq measurement TCs | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103724 | 0168 | - | LTE-RRM:CR to E-UTRAN TDD RRC Re-establishment test cases | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103734 | 0169 | - | Test Tolerances and alignment for RLM FDD TC 7.3.1, 7.3.2 | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103736 | 0170 | - | Uncertainties and Test Tolerances for Inter Frequency Absolute RSRP Accuracy | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103737 | 0171 | - | Uncertainties and Test Tolerances for TC 8.1.3 and 8.2.2 | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103738 | 0172 | - | Uncertainties and Test Tolerances for TC 8.4.1 and 8.4.2 | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103739 | 0173 | - | LTE-RRM: CR for Test Tolerances of intra-freq hand over test cases (5.1.1 & 5.1.2) | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103740 | 0174 | - | LTE-RRM:CR for Test Tolerances of inter-freq absolute RSRQ accuracy test cases (9.2.3.1 & 9.2.4.1) | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103741 | 0175 | - | LTE-RRM:CR for Test Tolerances of inter-freq relative RSRQ accuracy test cases (9.2.3.2 & 9.2.4.2) | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103742 | 0176 | - | Uncertainties and Test Tolerances for Inter Frequency Relative RSRP Accuracy | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103743 | 0177 | - | LTE-RRM: CR on Test Tolerances for TDD intra-freq absolute RSRP accuracy Test | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103744 | 0178 | - | LTE-RRM: CR on Test Tolerances for TDD intra-freq relative RSRP accuracy Test case | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103745 | 0179 | - | Addition of test tolerances and system uncertainties for E-UTRAN TDD-TDD HO inter-frequency case | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103746 | 0180 | - | Additions to measurement uncertainties and Test Tolerances for E-UTRAN FDD-FDD and TDD-TDD HO inter-frequency case in Annex F | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103747 | 0181 | - | Additions to measurement uncertainties and Test Tolerances for E-UTRAN FDD-FDD and TDD-TDD intra frequency cell search in Annex F | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103748 | 0182 | - | Addition of test tolerances and system uncertainties for FDD intra frequency absolute RSRQ accuracy | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103749 | 0183 | - | Addition of test tolerances and system uncertainties for TDD intra frequency absolute RSRQ accuracy | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103750 | 0184 | - | Additions to measurement uncertainties and Test Tolerances for FDD and TDD intra frequency absolute RSRQ accuracy in Annex F | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103758 | 0185 | - | CR on 36.521-3 for corrections of missing Es/Noc parameters in RRM test cases | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103759 | 0186 | - | Adding new test case 8.11.5 Combined E-UTRAN - EUTRAN FDD and GSM cell search | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103760 | 0187 | - | Adding new test case 8.11.6 Combined E-UTRAN - EUTRAN TDD and GSM cell search. | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103761 | 0188 | - | Adding test case 8.7.3, 8.11.5, 8.11.6 to Annex E Cell configuration mapping. | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103769 | 0189 | - | Adding band 20, 800MHz in EU to TS36.521-3 | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103773 | 0190 | - | Iteration in Handover and Re-establishment test cases | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103779 | 0191 | - | LTE-RRM: Addition of new TC E-UTRAN FDD -UTRAN FDD CPICH Ec/No absolute accuracy | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103783 | 0192 | - | Correction to q-RxLevMin for E-UTRAN - GSM cell re-selection | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103784 | 0145 | - | DL Mac Padding for RRM TCs | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103105 | 0146 | - | CR to 36.521-3: Update RSRP test cases with band 11 and 21 | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103116 | 0147 | - | Correction of CR conflict for Intra Frequency TDD reselection test | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103117 | 0201 | - | Reference to FTR 36.903 in 3GPP TS 36.521-3 | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103312 | 0150 | - | Connection diagram for test 8.11.2 (3 cells) | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103315 | 0151 | - | Correction to connection diagram reference for test 8.10.1 and 8.10.2 | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103330 | 0152 | - | update on test applicability | 9.0.0 | 9.1.0 |
| 2010-06 | RAN#48 | R5-103358 | 0153 | - | Annex E update | 9.0.0 | 9.1.0 |
| 2010-09 | RAN#49 | R5-104098 | 0198 | - | PUSCH Scheduling for RRM tests | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104103 | 0199 | - | Delay exclusion for retransmissions in RRM test cases | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104108 | 0200 | - | Expiry of contention resolution timer in Contention based PRACH test | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104160 | 0201 | - | Uncertainties and Test Tolerances for FDD Intra Frequency Relative RSRP Accuracy clause 9.1.1.2 | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104230 | 0202 | - | Correction CR to 36.521-3: Update of E-UTRAN TDD-TDD cell re-selection intra frequency case 4.2.2 | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104231 | 0203 | - | Correction CR to 36.521-3: Update of E-UTRAN TDD-TDD cell re-selection inter frequency case 4.2.6 | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104232 | 0204 | - | Correction CR to 36.521-3: Update of E-UTRAN TDD-TDD Handover intra frequency case 5.1.2 | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104233 | 0205 | - | Correction CR to 36.521-3: Update of E-UTRAN TDD-TDD Handover inter frequency case 5.1.4 | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104247 | 0206 | - | Addition of Cell Configuration Mapping for Cell Search Test | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104248 | 0207 | - | CR to 36.521-3 on Correction to cell search | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104249 | 0208 | - | CR to 36.521-3 on Correction to UE Measurement Procedures | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104250 | 0209 | - | CR to 36.521-3 on Correction to RRM Cell Search | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104251 | 0210 | - | CR to 36.521-3 on Correction to RRM General | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104260 | 0211 | - | Addition of test tolerances and system uncertainties for E-UTRAN FDD random access tests | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104261 | 0212 | - | Addition of test tolerances and system uncertainties for E-UTRAN TDD random access tests | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104262 | 0213 | - | LTE-RRM: Addition of new TC E-UTRAN TDD -UTRAN FDD CPICH Ec/No absolute accuracy | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104263 | 0214 | - | LTE-RRM: Addition of new TC E-UTRAN TDD -UTRAN FDD CPICH RSCP absolute accuracy | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104451 | 0215 | - | Test Tolerances and alignment for RLM FDD TC 7.3.3, 7.3.4 | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104452 | 0216 | - | Test Tolerances and alignment for RLM FDD TC 7.3.5, 7.3.6 | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104453 | 0217 | - | Test Tolerances and alignment for RLM TDD TC 7.3.7, 7.3.8 | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104456 | 0218 | - | Uncertainties and Test Tolerances for E-UTRAN FDD Intra-frequency RRC Re-establishment | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104460 | 0219 | - | Uncertainties and Test Tolerances for E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104497 | 0220 | - | Clarification on the neighbour cell info | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104498 | 0221 | - | Addition of the exceptional message to UE Transmit Timing | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104499 | 0222 | - | Maintenance on exceptional messages for annex info | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104500 | 0223 | - | Correction to 6.1.1 and 6.1.2 of RRC Re-establishment test case | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104501 | 0224 | - | Maintenance on exceptional messages for Mobility Control Info | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104521 | 0225 | - | 36521-3 General update of clauses 00 to 07: missing Introduction references formatting | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104563 | 0226 | - | Update on exclusion of extra delay due to RRC retransmission | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104616 | 0227 | - | 36.521-3 Correction to test procedure in 8.11.5 and 8.11.6 test cases | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104625 | 0228 | - | E-UTRAN TDD inter-frequency reselection test | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104650 | 0229 | - | Clarifications of test requirements in measurement accuracy tests | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104663 | 0230 | - | 36.521-3: Annex B and Annex C update | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104825 | 0231 | - | Missing cell Identity change step for test cases with unknown cell 2 timing | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104826 | 0232 | - | Addition of test tolerances and system uncertainties for FDD timing characteristics tests | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104827 | 0233 | - | Addition of test tolerances and system uncertainties for TDD timing characteristics tests | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104828 | 0234 | - | Additions to measurement uncertainties and test tolerances for timing characteristics tests in annex F | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104829 | 0235 | - | Uncertainties and Test Tolerances for E-UTRAN FDD Inter-frequency RRC Re-establishment | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104830 | 0236 | - | Uncertainties and Test Tolerances for E-UTRAN TDD-TDD inter-frequency Handover with unknown target cell | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104839 | 0237 | - | 36521-3: Editorial update of clauses 08 | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104849 | 0238 | - | Maintenance on the exceptional messages in Ch8 - Annex | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104855 | 0239 | - | Uncertainties, Test Tolerances and Test Requirements for UE Transmit Timing | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104856 | 0240 | - | GSM carrier RSSI measurement accuracy in E-UTRAN TDD | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104859 | 0241 | - | E-UTRAN_to_UTRAN_FDD_reselection | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104864 | 0242 | - | Applicability of RRM inter-frequency test cases to (narrow) frequency bands | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104865 | 0243 | - | Maintenance on the exceptional messages in Ch5 - Ch6 | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104866 | 0244 | - | 36.521-3: Annex E update | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104880 | 0245 | - | Correction to E-UTRAN to UTRAN Cell Re-Selection test case | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104881 | 0246 | - | Redundant information in RRM Random Access Test Requirements | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104883 | 0247 | - | E-UTRAN TDD to UTRAN FDD Handover | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104885 | 0248 | - | Cell ID change time and iteration procedure for RRM test cases 4.2.1, 4.2.2 | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104886 | 0249 | - | Cell ID change time for RRM test cases 4.2.3, 4.2.6 | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104887 | 0250 | - | Scrambling code change time for RRM test cases 4.3.1.1, 4.3.4.1, 8.5.2, 8.7.3, 8.11.4 | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104889 | 0251 | - | Iteration procedure for handover and re-establishment test cases | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-104890 | 0252 | - | Correction to cell re-selection inter frequency test case | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | R5-105057 | 0253 | - | Clarification of Radio link monitoring test cases | 9.1.0 | 9.2.0 |
| 2010-09 | RAN#49 | RP-100941 | 0254 | - | Correction of status for RRM test cases and missing information in Annex | 9.1.0 | 9.2.0 |
| - | - | - | - | - | Re-insertion of the ambiguous step 11 of cl. 5.2.2.4.2 according to R5-104825 after email discussion | 9.2.0 | 9.2.1 |
| 2010-12 | RAN#50 | R5-106079 | 0255 | - | HARQ delay exclusion for HO test: Clarification for UE-DTX-case | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106080 | 0256 | - | Iteration procedure for inter RAT handover test cases | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106082 | 0257 | - | Corrections to event triggered measurement tests using DRX (Clause 8) | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106083 | 0258 | - | Missing titles in the RRM specification | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106085 | 0259 | - | Scheduling of System information for RRM tests | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106086 | 0260 | - | Update of PDCCH aggregation level for channel BW 1,4 MHz | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106119 | 0261 | - | CR to 36.521-3: Update LTE RRM test requirements for EUTRA TDD LTE band 41. | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106313 | 0262 | - | Uncertainties and Test Tolerances for Connected State Mobility test | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106314 | 0263 | - | Addition to Measurement Uncertainties and Test Tolerances for Connected State Mobility Test in Annex | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106318 | 0264 | - | Correction to inter-RAT Connected State Mobility test setup | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106320 | 0265 | - | Correction to Inter-RAT Connected State Mobility test requirements | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106321 | 0266 | - | Correction to Inter-RAT Connected State Mobility for Alignment | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106322 | 0267 | - | Correction to Inter-RAT Connected State Mobility test requirements | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106448 | 0268 | - | Addition of SIB7 exceptional messages | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106451 | 0269 | - | Correction to UE transmit timing TC | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106455 | 0270 | - | Correction to the exceptional messages in RSRQ tests | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106456 | 0271 | - | Correction to Min Test time for RRM fading tests | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106483 | 0272 | - | Annex E update | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106493 | 0273 | - | CR to 36.521-3: Update to G.2.6 Test Conditions for Delay Tests and UE Measurement Performance | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106805 | 0274 | - | Correction to test case 5.1.2 - Update of E-UTRAN TDD-TDD Handover intra frequency case | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106806 | 0275 | - | Correction to test case 5.1.4 - Update of E-UTRAN TDD-TDD Handover inter frequency case | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106807 | 0276 | - | Correction to Inter-RAT UE Measurements Procedures | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106808 | 0277 | - | Correction to Inter-RAT UE Measurements Procedures under fading | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106810 | 0278 | - | Correction to test case 8.2.1 | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106811 | 0279 | - | Correction to test case 8.2.2 | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106812 | 0295 | - | Update of RRM OCNG patterns | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106829 | 0280 | - | General Corrections to RRC_IDLE State Mobility | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106830 | 0281 | - | Correction to test case 6.2.3 | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106831 | 0282 | - | Correction to test case 6.2.4 | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106832 | 0283 | - | Correction to MeasConfig-DEFAULT in RRM TCs | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106833 | 0284 | - | Adding support of inter-band test configuration for RRM inter-frequency/inter-RAT test cases | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106834 | 0285 | - | CR on UEs RRM Band applicability | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106835 | 0286 | - | Correction to test case 7.1.2 | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106836 | 0287 | - | Correction to test case 10.1.2.1, 9.1.2.2 and 9.2.2.1 | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106840 | 0288 | - | Update to Radio Link Monitoring Test Cases | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106857 | 0291 | - | Correction to DL configuration on Non-Contention Based Random Access Test | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106859 | 0292 | - | Corrections to UE transmit timing tests (Subclause 7.3) | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106862 | 0293 | - | Correction to DL configuration on Contention Based Random Access Test | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106864 | 0294 | - | Update of Annex G for RLM test in DRX | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106870 | 0289 | - | Uncertainties and Test Tolerances for UE measurements procedures test | 9.2.1 | 9.3.0 |
| 2010-12 | RAN#50 | R5-106871 | 0290 | - | Addition to Measurement Uncertainties and Test Tolerances for UE Measurement Procedures test in Annex | 9.2.1 | 9.3.0 |
| 2011-03 | RAN#51 | R5-110150 | 0296 | - | RRC Re-establishment tests: Corrections to Message contents | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110151 | 0297 | - | Radio link monitoring tests: Corrections to Message contents | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110155 | 0298 | - | UE Measurements Procedures tests: Test loop | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110167 | 0299 | - | Removal of [ ] from PDSCH and PCFICH/PDCCH/PHICH Measurement Channel references | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110348 | 0300 | - | Revision of 36.521-3 Annex G - Statistical testing | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110418 | 0301 | - | Correction to TDD cell re-selection | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110419 | 0302 | - | Correction to exception messages in 4.5.1 HRPD Re selection test | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110424 | 0303 | - | Alignment of exception messages for TDD event triggered measurement tests | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110435 | 0304 | - | Modification of message content definition for TC 8.4.1 | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110437 | 0305 | - | Update to TC 8.6.1: E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110438 | 0306 | - | Correction to TC 8.7.1: E-UTRA TDD to UTRA(1.28 Mcps TDD OPTION) cell search in fading propagation conditions | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110443 | 0307 | - | Update to TC 8.8.1: E-UTRAN FDD - GSM event triggered reporting in AWGN | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110445 | 0308 | - | Corrections to TC 8.9.1: E-UTRAN FDD - UTRAN TDD event triggered reporting under fading propagation conditions | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110520 | 0309 | - | Correction to Inter-RAT Connected State Mobility for Alignment | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110546 | 0310 | - | Uncertainties and Test Tolerances for Connected State Mobility Inter-RAT to UTRAN test | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110549 | 0312 | - | Uncertainties and Test Tolerances for Connected State Mobility Inter-RAT to GSM unknown test | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110584 | 0314 | - | Correction to gap pattern ID in test case 5.1.4 | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110586 | 0315 | - | Clarification to 1.4 MHz testing and applicability in test case 7.1.1 | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110588 | 0316 | - | Test time limit correction for DRX=40ms in test case 8.1.3 | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110863 | 0330 | - | Higher SNR on event triggered measurement tests | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110866 | 0311 | - | Addition to Measurement Uncertainties and Test Tolerances for Connected State Mobility Inter-RAT to UTRAN test in Annex | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110868 | 0313 | - | Addition to Measurement Uncertainties and Test Tolerances for Connected State Mobility Inter-RAT to GSM unknown test in Annex | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110902 | 0317 | - | CR to 36.521-3: Update LTE RRM test requirements for EUTRA TDD LTE band 41 | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110903 | 0318 | - | Correction to exception messages in 5.3.1 HRPD HHO test | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110904 | 0319 | - | MIMO Correlation scenario for RLM test cases | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110905 | 0320 | - | Enabling HARQ for clause 8 and 9 RRM Tests | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110907 | 0321 | - | Re-ordering of Time periods, definition of uncertainties, and addition of Test Tolerances for RRM test case 4.3.1.1 | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110910 | 0322 | - | Updated Test Tolerances for RRM Test cases 7.1.1 + 7.1.2 | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110911 | 0323 | - | Uncertainties and Test Tolerances for Connected State Mobility Inter-RAT to GSM tests | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110912 | 0324 | - | Addition to Measurement Uncertainties and Test Tolerances for Connected State Mobility Inter-RAT to GSM test in Annex | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110927 | 0325 | - | Corrections to RRM TC 8.1.1, 8.1.2 and 8.1.3 | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110928 | 0326 | - | Corrections to test cases about E-UTRAN FDD-FDD Inter-frequency measurement 8.3.1, 8.3.2 and 8.3.3 | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110929 | 0327 | - | Corrections to TCs related to E-UTRAN FDD - UTRAN measurements: 8.5.1, 8.5.2 and 8.5.3 | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110930 | 0328 | - | UE Measurement procedures tests: Corrections to Message contents | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110931 | 0329 | - | DL-RMC-s and OCNG for RRM tests: Updates | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110946 | 0331 | - | Uncertainties and Test Tolerances for RRM test case 4.3.1.2 | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110948 | 0332 | - | Uncertainties and Test Tolerances for RRM test cases 4.4.1 and 4.4.2 | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110956 | 0333 | - | Modification of test case 5.1.6 - E-UTRAN TDD-TDD inter frequency handover: unknown target cell | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110957 | 0334 | - | LTE RRM: reference to state 3A in 36.521-3 | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110958 | 0335 | - | Correction to RRM testes for Alignment | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110959 | 0336 | - | CR to 36.521-3: E-UTRAN FDD - UTRA FDD CPICH RSCP absolute accuracy test case | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110960 | 0337 | - | CR to 36.521-3: Addition of Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting test case | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110961 | 0338 | - | CR to 36.521-3: E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting test case | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110962 | 0339 | - | Correction to exception messages in Radio Link Monitoring Test | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110963 | 0340 | - | Correction to TC 8.4.2: E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110964 | 0341 | - | Corrections to TC 8.7.3: E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting under AWGN propagation conditions | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110965 | 0342 | - | Correct the message definitions related to the RSRP and RSRQ performance testing | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110966 | 0343 | - | Update of RRM test 8.5.2 FDD SON | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110974 | 0344 | - | PUSCH scheduling: Correction for considering DRX | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110980 | 0345 | - | Correction to TC 8.8.2: E-UTRAN FDD - GSM Event triggered reporting when DRX is used in AWGN | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110981 | 0346 | - | Update to TC 8.10.1: E-UTRAN TDD - GSM event triggered reporting in AWGN | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110982 | 0347 | - | Corrections to TC 8.10.2: E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110983 | 0348 | - | Modification to TC 8.7.2: E-UTRAN TDD to UTRAN 1.28Mcps TDD cell search when DRX is used in fading propagation conditions | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110995 | 0352 | - | Radio link monitoring test 7.3.4: Minor correction to the test requirement | 9.3.0 | 9.4.0 |
| 2011-03 | RAN#51 | R5-110996 | 0353 | - | Radio link monitoring tests: Corrections to the test procedure | 9.3.0 | 9.4.0 |
| 2011-06 | RAN#52 | R5-112124 | 0354 | - | Uncertainties and Test Tolerances for RRM test case 8.7.1 | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112126 | 0355 | - | Uncertainties and Test Tolerances for RRM test case 8.7.2 | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112128 | 0356 | - | Uncertainties and Test Tolerances for RRM test cases 8.8.1+8.10.1 | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112152 | 0357 | - | RRM TC-s 4.2: Transition between time intervals | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112153 | 0358 | - | RRM TC 4.2.6: Introduction of time duration T0 | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112155 | 0359 | - | RRM TC-s clause 8: Reference to the state 3A / 3A-RF in the test loop | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112185 | 0360 | - | Addition of new RRM TC 4.3.4.3: EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112314 | 0365 | - | Correction to E-UTRAN FDD - UTRAN FDD cell re-selection when UTRA FDD is under lower priority | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112315 | 0366 | - | Correction to E-UTRA FDD-high UTRA FDD inter RAT cell re-selection test case | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112316 | 0367 | - | Correction to E-UTRAN TDD-UTRAN TDD cell search reporting for SON ANR in AWGN propagation conditions | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112317 | 0368 | - | Correction on test cases of E-UTRA to UTRA cell reselection in idle state | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112318 | 0369 | - | Correction to E-UTRAN TDD - UTRAN TDD test case in 36.521-3 | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112418 | 0370 | - | Update of 4.3.1.3 E-UTRA-UTRA reselection test case | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112421 | 0371 | - | Correction to 6.1 RRC Re-establishment test case | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112423 | 0372 | - | Maintenance on Message contents in 8.5.3 | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112424 | 0373 | - | Correction to Annex H.3.3 for Inter-RAT E-UTRAN - HRPD handover | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112454 | 0374 | - | Wrong references into statistical annex | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112457 | 0375 | - | References into connection diagrams in 36.508,Annex A | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112470 | 0376 | - | Misalignment in Meas Gap configuration | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112475 | 0377 | - | Band 20 missing in clause 9 test cases | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112533 | 0378 | - | Uncertainties and Test Tolerances for connected state mobility Inter-RAT TDD to GSM test | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112536 | 0379 | - | Addition to Measurement Uncertainties and Test Tolerances for connected state mobility Inter-RAT TDD to GSM test in Annex | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112543 | 0380 | - | Uncertainties and Test Tolerances for connected state mobility Inter-RAT TDD to GSM unknown test | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112544 | 0381 | - | Addition to Measurement Uncertainties and Test Tolerances for connected state mobility Inter-RAT TDD to GSM unknown test in Annex | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112546 | 0382 | - | Uncertainties and Test Tolerances for Inter-RAT to UTRAN event triggered reporting under fading test | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112554 | 0383 | - | Uncertainties and Test Tolerances for Inter-RAT to GSM event triggered when DRX is used in AWGN tests | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112555 | 0384 | - | Addition to Measurement Uncertainties and Test Tolerances for Inter-RAT to GSM event triggered when DRX is used in AWGN tests in Annex | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112734 | 0385 | - | Addition of Band 24 to clause 9.1 and 9.2 , RSRP and RSRQ measurement performance requirements | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112741 | 0363 | - | Uncertainties and Test Tolerances for RRM test case 10.3.1 | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112742 | 0364 | - | Uncertainties and Test Tolerances for RRM test case 9.3.2 | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112745 | 0394 | - | Completing for E-UTRAN TDD-UTRAN TDD cell reselection_UTRA is of lower priority | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112746 | 0395 | - | Completing for E-UTRAN FDD-UTRAN FDD-UTRAN TDD cell reselection | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112803 | 0386 | - | Addition to Measurement Uncertainties and Test Tolerances for Inter-RAT to UTRAN event triggered reporting under fading test in Annex | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112815 | 0387 | - | Correction to test frequency references in RRM initial condition | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112817 | 0388 | - | RRM TC-s 8.1.1, 8.1.2, 8.1.3, 8.2.1, 8.2.2: Changing SNR for serving cell | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112818 | 0389 | - | RRM TC 9.6.2: Overall corrections | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112819 | 0390 | - | CR for 9.4 UTRA FDD measurement accuracy | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112820 | 0391 | - | Add test frequencies for bands 42, 43 (3500MHz) | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112849 | 0398 | - | Update of clause 3A.3 RRM test configuration | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112853 | 0399 | - | Correction to inconsistent test procedures in RRM | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112855 | 0400 | - | Uncertainties and Test Tolerances for RRM test case 5.2.10 | 9.4.0 | 9.5.0 |
| 2011-06 | RAN#52 | R5-112858 | 0401 | - | Addition of new RRM TC 8.4.3: E-UTRAN TDD-TDD inter-freq event triggered reporting under AWGN in synchronous cells with DRX when L3 filtering is used | 9.4.0 | 9.5.0 |
| 2011-09 | RAN#53 | R5-113183 | 0402 | - | RRM TC 8: Adding missing PRACH Configuration for some tests | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-113226 | 0403 | - | Uncertainties and Test Tolerances for RRM test case 4.3.1.3 | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-113249 | 0404 | - | Uncertainties and Test Tolerances for TC 5.2.1 | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-113250 | 0405 | - | Uncertainties and Test Tolerances for TC 5.2.2 | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-113395 | 0406 | - | Not tested minimum requirement in Clause 8 | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-113460 | 0407 | - | Correction to 4.2.3 | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-113461 | 0408 | - | Correction to the exceptional messages in HO TCs with unknown target cell | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-113462 | 0409 | - | Maintenance on the exceptional messages for Mobility Control Info | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-113463 | 0410 | - | Correction to 6.2.3 and 6.2.4 | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-113466 | 0411 | - | Correction to FDD RSRP and RSRQ test | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-113467 | 0412 | - | Correction to TDD RSRP and RSRQ test for band 41 | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-113468 | 0413 | - | Correction to the exceptional messages in Annex H | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-113597 | 0414 | - | Abbreviation update and Editorial corrections in 3GPP TS36.521-3 | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-113843 | 0443 | - | Adding FGI Applicabilities into Chapters 4 - 7 in 36.521-3 | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-113844 | 0440 | - | RRM TCs 5.1: PRACH power configuration | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-113845 | 0444 | - | RRM TCs 7.3: Update of the test procedure and requirements | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-113846 | 0425 | - | Statistical clarification for TC 8.3.3 and 8.3.4 | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114005 | 0415 | - | LTE-RRM: Corrections to test iteration for test case 4.3.4.1 | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114007 | 0416 | - | Correction on the inter-RAT cell identification time in DRX | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114009 | 0417 | - | Completing for E-UTRAN TDD - UTRAN TDD handover test case | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114013 | 0418 | - | Uncertainties and Test Tolerance for FDD SON ANR test case 8.5.2 | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114016 | 0419 | - | Uncertainties and Test Tolerances for TC 9.4.1 and 9.4.2 | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114019 | 0420 | - | CR Uncertainties and TT for 8.4.3 in 36.521-3 | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114021 | 0421 | - | CR Uncertainties and TT for 4.3.4.3 in 36.521-3 | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114026 | 0422 | - | Deletion of editor note for discrepancy between TT and 36.903 | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114050 | 0423 | - | RRM: Use of State 3A-RF | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114055 | 0424 | - | RRM TCs 7.2: Transition between iteration loops | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114057 | 0426 | - | Statistical clarification in 6 Test cases in clause 8.11. | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114059 | 0427 | - | Completing for E-UTRAN TDD-UTRAN TDD cell re-selection_UTRA is of higher priority | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114060 | 0428 | - | Uncertainties and Test Tolerances for TC 8.9.1 | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114072 | 0429 | - | Update LTE RRM test requirements for FDD LTE Band 23 in 36.521-3 | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114084 | 0430 | - | Simplification of frequency dependent minimum requirements in 3GPP TS36.521-3 | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114097 | 0431 | - | Adding FGI Applicabilities into Chapters 8 - 9 in 36.521-3 | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114099 | 0432 | - | Addition of new RRM TC 8.1.5´╝ÜE-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114100 | 0433 | - | Addition of new RRM TC 8.1.6: E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114101 | 0434 | - | Addition of new RRM TC 8.2.3: E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114102 | 0435 | - | Addition of new RRM TC 8.2.4: E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114103 | 0436 | - | Addition of new RRM TC 8.3.4´╝ÜE-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114104 | 0437 | - | Addition of new RRM TC 8.3.5:E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114105 | 0438 | - | Addition of new RRM TC 8.4.4:E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114106 | 0439 | - | Addition of new RRM TC 8.4.5:E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114111 | 0441 | - | Correction to RLM | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114115 | 0442 | - | LTE-RRM: Correction to test procedure for inter-RAT cell reselection test cases | 9.5.0 | 9.6.0 |
| 2011-09 | RAN#53 | R5-114119 | 0445 | - | Introduction of Expanded 1900MHz Band (Band 25) into clause 9 of 36.521-3 | 9.5.0 | 9.6.0 |
| 2011-12 | RAN#54 | R5-115121 | 0446 | - | RRM TC-s 7, 8: Iteration loop and usage of the UE states 3A / 3A-RF | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115140 | 0447 | - | Modify the test requirement table in the TC 5.2.1 | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115142 | 0449 | - | LTE-RRM :Update to Annex E | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115189 | 0452 | - | Uncertainties and Test Tolerances for RRM test case 8.11.3 | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115199 | 0453 | - | Correction of references to Annex I in 3GPP TS36.521-3 | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115200 | 0454 | - | Test System uncertainties for frequencies between 3000MHz to 4200MHz in 36.521-3 | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115202 | 0456 | - | Uncertainties and Test Tolerance for TDD SON ANR test case 8.7.3 | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115327 | 0457 | - | Correction to RRM tests 7.1.2 and 7.2.2 | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115379 | 0458 | - | Update of operating band configuration | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115381 | 0459 | - | Correction to FGI in test applicability for Cell reselection test case | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115385 | 0460 | - | Correction to 5.2.2 | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115386 | 0461 | - | Alignment of the exceptional messages in 7.3.x RLM | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115387 | 0462 | - | Correction to 8.10.1 and 8.10.2 | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115388 | 0463 | - | Correction to the exceptional message in 8.6.1.1 | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115403 | 0465 | - | Uncertainties and Test Tolerances for RRM test case 4.3.3 | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115433 | 0466 | - | Corrections to TC 5.1.5 and TC 5.1.6 inter-f HO: unknown target cell | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115435 | 0467 | - | Updates of TC 5.2.3: E-UTRAN FDD - GSM handover | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115479 | 0468 | - | Corrections to RSRQ in Intra-Frequency Measurement Minimum Requirements | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115482 | 0469 | - | Addition to measurement uncertainties and test tolerances E-UTRAN FDD - UTRAN FDD event triggered reporting w/DRX under fading test in Annex | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115787 | 0471 | - | Uncertainties and Test Tolerances for RRM test case 8.11.1 and 8.11.2 | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115582 | 0472 | - | RRM TC 6.2: Corrections to power settings | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115814 | 0473 | - | Incomplete test case for 7.1.1 and 7.1.2 | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115823 | 0474 | - | Uncertainties and TT for TC 6.1.3 and 6.1.4 in 36.521-3 | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115827 | 0477 | - | Correction to Test Tolerances for RRM ch.9 test cases | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115833 | 0478 | - | Adding band 22 (3500MHz FDD) to 36.521-3 | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115834 | 0479 | - | RRM: Phase rotation for intra frequency tests in static conditions | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115835 | 0480 | - | Addition of the exceptional message in 4.6.1.1 | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115836 | 0481 | - | Addition of undefined UTRA system information for TC 4.3.1.1 | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115838 | 0482 | - | Corrections to TC 5.2.1 and TC 5.2.2: E-UTRAN FDD/TDD - UTRAN FDD handover | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115839 | 0483 | - | Corrections to message content definition for TC 5.1.1 and TC 5.1.2: intra-f HO | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115840 | 0484 | - | Updates to TC 5.1.3 and TC 5.1.4: inter-f HO | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115841 | 0485 | - | Updates of TC 5.2.4 and TC 5.2.5: E-UTRAN FDD/TDD - UTRAN TDD handover | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115842 | 0486 | - | Removal of measurement related message definitions in TC 5.2.7, TC 5.2.9 and TC 5.2.10 | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115843 | 0487 | - | Modification of message definitions in the Annex H | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115844 | 0488 | - | Modification of the test cases of Random Access | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115845 | 0489 | - | RRM TC-s 9: Missing bands in specification | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115846 | 0490 | - | RRM TC-s 7.3: SRS configuration in radio link monitoring tests | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115847 | 0491 | - | Correction to test frequency in MeasConfig-DEFAULT | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115848 | 0492 | - | Corrections to TC 7.1.1 and TC 7.1.2: UE Transmit Timing Accuracy | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115850 | 0493 | - | Correction to 5.2.6 | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115878 | 0494 | - | Correction to cell reselection delay in test procedure | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115882 | 0495 | - | Addition of undefined UTRA system information for TC 4.3.1.2 and TC 4.3.1.3 | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115883 | 0496 | - | Addition of undefined UTRA system information for TC 4.3.2 | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115884 | 0497 | - | Addition of undefined UTRA system information for TC 4.3.3 | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115885 | 0498 | - | Addition of undefined UTRA system information for TC 4.3.4.1 | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115886 | 0499 | - | Addition of UTRA system information definitions for TC 4.3.4.2 | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115887 | 0500 | - | Updates of the message content definitions for TC 4.3.4.3 | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115888 | 0501 | - | Updates of TC 4.4.1 and TC 4.4.2: E-UTRAN FDD/TDD - GSM cell re-selection | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115786 | 0502 | - | Corrections to test cases for E-UTRAN RRC Re-establishment | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115893 | 0503 | - | RRM TC-s 4: General review of the test procedures of cell re-selection test cases | 9.6.0 | 9.7.0 |
| 2011-12 | RAN#54 | R5-115481 | 0504 | - | Uncertainties and test tolerances E-UTRAN FDD - UTRAN FDD event triggered reporting w/DRX under fading test | 9.6.0 | 9.7.0 |
| 2012-03 | RAN#55 | R5-120107 | 0505 | - | Uncertainties and Test Tolerance for E-UTRAN TDD Intra-frequency new CGI test cases 8.2.3 and 8.2.4 | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120124 | 0506 | - | Uncertainties and Test Tolerance for E-UTRAN TDD Inter-frequency new CGI test cases 8.4.4 and 8.4.5. | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120141 | 0507 | - | RRM: Iteration loop in cdma2000 reselection tests | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120178 | 0508 | - | RF/RRM: Correction on TC 8.4.1 message content definition | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120183 | 0509 | - | RF/RRM: Addition of new TC 4.2.4 E-UTRAN FDD - TDD cell re-selection inter frequency case | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120184 | 0510 | - | RF/RRM: Addition of new TC 4.2.5 E-UTRAN TDD - FDD cell re-selection inter frequency case | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120185 | 0511 | - | RF/RRM: Addition of new TC 5.1.7 E-UTRAN FDD - TDD handover inter frequency case | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120186 | 0512 | - | RF/RRM: Addition of new TC 5.1.8 E-UTRAN TDD - FDD handover Inter frequency case | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120187 | 0513 | - | RF/RRM: Addition of new TC 8.12.1 E-UTRAN TDD - FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120189 | 0514 | - | RF/RRM: Addition of new TC 9.1.5.1 FDD - TDD Inter Frequency Absolute RSRP Accuracy | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120190 | 0515 | - | RF/RRM: Addition of new TC 9.1.5.2 FDD - TDD Inter Frequency Relative Accuracy of RSRP | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120191 | 0516 | - | RF/RRM: Addition of Cell configuration mapping for those new RRM test cases | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120245 | 0517 | - | Update of 36.521-3 Test Cases 9.1.4.1 and 9.1.4.2, Io difference band-independent | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120249 | 0518 | - | Uncertainties and Test Tolerances for RRM test case 8.11.4 | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120321 | 0519 | - | Update of 4.3.1 E-UTRAN FDD - UTRAN FDD cell re-selection | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120322 | 0520 | - | Update of 4.3.4 E-UTRAN TDD - UTRAN TDD cell re-selection | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120323 | 0521 | - | Adding E-UTRAN test parameter reference to messages exception | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120324 | 0522 | - | Correction to 4.6.1.1 E-UTRAN FDD c2k cell re-selection | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120325 | 0523 | - | Correction to 5.2.4 E-UTRAN TDD - UTRAN TDD handover | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120339 | 0524 | - | Addition of FGI bit 16 into test cases 9.1.x.x and 9.2.x.x | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120341 | 0525 | - | Correction to FGI bits in test case 8.5.2 | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120424 | 0526 | - | Correction to TIntra in Minimum Conformance Requirements | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120425 | 0527 | - | Correction to the identification time in DRX for UTRA TDD | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120515 | 0528 | - | Addition of FGI bit 15 into test cases configuring event 1B | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120808 | 0535 | - | RF/RRM: Corrections on RSRP and RSRQ accuracy related test cases | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120809 | 0536 | - | TS 36.521-3: 8.3.3 and 8.4.3 T2 value correction | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120810 | 0537 | - | TS 36.521-3: 6.2.3 Extreme conditions test tolerance correction | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120827 | 0538 | - | Correction to CQI report configuration of 7.1.1 in 3GPP TS36.521-3 | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120846 | 0539 | - | Test configuration for Inter RAT testcases, delete note | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120847 | 0540 | - | RF/RRM: Addition of new TC 8.13.1 E-UTRAN FDD - TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120848 | 0541 | - | Correction to 8.7.3 E-UTRAN TDD - UTRAN TDD SON ANR | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120892 | 0542 | - | Update of PRACH test case | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120894 | 0543 | - | Correction to 5.2.10 E-UTRAN TDD - UTRAN TDD handover | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120895 | 0544 | - | Correction to test frequency in MeasConfig-DEFAULT for E-UTRAN to GSM cell search test case | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120907 | 0547 | - | Uncertainties and Test Tolerances for E-UTRAN to HRPD Cell reselection TC 4.5.1.1 | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120917 | 0546 | - | Uncertainties and Test Tolerances for E-UTRAN to HRPD HO TC 5.3.1 | 9.7.0 | 9.8.0 |
| 2012-03 | RAN#55 | R5-120530 | 0529 | - | Introduction to FDD RSRQ for E-UTRA Carrier Aggregation | 9.8.0 | 10.0.0 |
| 2012-03 | RAN#55 | R5-120531 | 0530 | - | Introduction to TDD RSRQ for E-UTRA Carrier Aggregation | 9.8.0 | 10.0.0 |
| 2012-03 | RAN#55 | R5-120532 | 0531 | - | Introduction to FDD RSRQ for E-UTRA Carrier Aggregation in Annex | 9.8.0 | 10.0.0 |
| 2012-03 | RAN#55 | R5-120533 | 0532 | - | Introduction to TDD RSRQ for E-UTRA Carrier Aggregation in Annex | 9.8.0 | 10.0.0 |
| 2012-03 | RAN#55 | R5-120535 | 0533 | - | Introduction to Carrier Aggregation in Radio Resource Management | 9.8.0 | 10.0.0 |
| 2012-03 | RAN#55 | R5-120536 | 0534 | - | Introduction to Carrier Aggregation in Default Message Contents | 9.8.0 | 10.0.0 |
| 2012-06 | RAN#56 | R5-121229 | 0549 | - | Correction of test procedures for Autonomous gap test cases, clause 8 | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-121240 | 0550 | - | Revise test frequencies for FDD-TDD interworking Test cases | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-121247 | 0551 | - | RRM: Removal of Editors note on connection diagram used in intra frequency tests in static conditions | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-121527 | 0552 | - | Addition of Handover to UTRAN commands in 36.521-3 | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-121528 | 0553 | - | Correction of drx-RetransmissionTimer parameters | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-121529 | 0554 | - | Correction to Test2 in 7.1.1 and 7.1.2 of 36.521-3 | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-121530 | 0555 | - | Correction to DRX offset in 7.1.2 | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-121532 | 0557 | - | Correction to 8.11.1 and 8.11.2 | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-121547 | 0558 | - | Addition of new TCs for UTRAN TDD P-CCPCH RSCP measurement | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-121691 | 0559 | - | TS 36.521-3: 8.3.3 and 8.4.3 update | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-121901 | 0560 | - | Introduction of E-UTRAN Inter Introduction of E-UTRAN Inter frequency case reselection in the existence of non-allowed CSG cell | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-121902 | 0561 | - | Addition of new RRM TC 6.3.1: Redirection from E-UTRAN FDD to UTRAN FDD | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-121922 | 0562 | - | Addition of new RRM TC 6.3.2: Redirection from E-UTRAN TDD to UTRAN FDD | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-121923 | 0563 | - | Addition of new RRM TC 6.3.3: Redirection from E-UTRAN FDD to GERAN when System Information is provided | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-121924 | 0564 | - | Addition of new RRM TC 6.3.4: Redirection from E-UTRAN TDD to GERAN when System Information is provided | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-121927 | 0565 | - | Uncertainties and Test Tolerances for TC 9.6.2 | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-121929 | 0566 | - | Uncertainties and Test Tolerances for RRM test cases 4.2.4 and 4.2.5 | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-121930 | 0567 | - | Uncertainties and Test Tolerances for RRM test cases 5.1.7 and 5.1.8 | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-121931 | 0568 | - | Uncertainties and Test Tolerances for RRM test cases 8.14.1 | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-121932 | 0569 | - | Uncertainties and Test Tolerances for RRM test case 8.15.1 | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-121933 | 0570 | - | Uncertainties and Test Tolerances for RRM test case 9.1.5.1 | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-121934 | 0571 | - | Uncertainties and Test Tolerances for FDD - TDD Inter Frequency Relative Accuracy of RSRP test case 9.1.5.2 | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-121940 | 0572 | - | Correction to the test mode references for RRM tests | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-121941 | 0573 | - | Addition of new RRM TC 6.3.5: E-UTRA TDD RRC connection release redirection to UTRA TDD | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-121942 | 0574 | - | RRM: Introduction of Annex for handling of different releases and UE capabilities | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-121969 | 0575 | - | Adding operating band 26 to TS 36.521-3 | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-121977 | 0576 | - | Change in transmit timing tests based on DRX feature group indicator | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-121980 | 0577 | - | Clarifications to FDD RSRQ for E-UTRA Carrier Aggregation | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-121981 | 0578 | - | Clarifications to TDD RSRQ for E-UTRA Carrier Aggregation | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-121991 | 0579 | - | Further Test Tolerance analysis for operating band 25 and 41 in 36.521-3 | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-121998 | 0580 | - | Addition of new RRM TC 6.3.6: E-UTRA FDD RRC connection release redirection to UTRA TDD | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-121999 | 0581 | - | Addition of new RRM TC 8.5.4 E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-122000 | 0582 | - | Addition of new RRM TC 8.7.4 E-UTRA TDD-UTRA TDD enhanced cell identification under AWGN propagation conditions | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-122001 | 0583 | - | Addition of new RRM TC 8.9.2 E-UTRA FDD-UTRA TDD enhanced cell identification under AWGN propagation conditions | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-122002 | 0584 | - | RRM: Clarifications to the OCNG patterns | 10.0.0 | 10.1.0 |
| 2012-06 | RAN#56 | R5-122007 | 0556 | - | Correction to SR Config Index for TDD DRX test cases | 10.0.0 | 10.1.0 |
| 2012-09 | RAN#57 | R5-123065 | 0585 | - | Correction to References in Annex I | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123149 | 0587 | - | Corrections to E-UTRAN FDD intra frequency measurements requirements | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123151 | 0588 | - | Corrections to E-UTRAN FDD inter frequency measurements requirements | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123152 | 0589 | - | Corrections to E-UTRAN TDD intra frequency measurements requirements | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123153 | 0590 | - | Corrections to E-UTRAN TDD inter frequency measurements requirements | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123163 | 0591 | - | Introduction of E-UTRAN TDD-TDD Event Triggered reporting on deactivated SCell with PCell interruption in non-DRX for CA | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123166 | 0592 | - | Correction to FDD RSRQ for E-UTRA Carrier Aggregation tests | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123167 | 0593 | - | Correction to TDD RSRQ for E-UTRA Carrier Aggregation tests | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123168 | 0594 | - | Introduction of default RRC messages exceptions for Carrier Aggregation | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123281 | 0596 | - | Addition of new TC 4.6.2.1 E-UTRAN TDD-cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123290 | 0597 | - | Addition of new TC 8.18.1 E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123291 | 0598 | - | Addition of new TC 8.19.1 E-UTRAN TDD-CDMA2000 1X event triggered reporting under fading propagation conditions | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123292 | 0599 | - | Annex H message content updates | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123301 | 0600 | - | Addition of Cell configuration mapping for new RRM test cases | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123335 | 0601 | - | Correction to transmit timing test cases | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123336 | 0602 | - | Addition of band indicator for GERAN | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123426 | 0603 | - | RRM: Update of Annex J | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123903 | 0606 | - | Addition of new RRM TC 4.5.2.1 E-UTRAN TDD-HRPD Cell Reselection: HRPD is of Lower Priority | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123904 | 0607 | - | Addition of new TC 5.3.5 E-UTRAN TDD-HRPD Handover | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123905 | 0608 | - | Addition of new TC 6.3.8 E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123906 | 0609 | - | Addition of new TC 6.3.9 Redirection from E-UTRAN FDD to UTRAN FDD without System Information | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123907 | 0610 | - | Addition of new TC 6.3.10 Redirection from E-UTRAN FDD to GERAN when System Information is not provided | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123908 | 0611 | - | Addition of new TC 6.3.11 Redirection from E-UTRAN TDD to GERAN when System Information is not provided | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123916 | 0612 | - | Introduction of Chapter9 absolute and relative RSRP measurement test cases for carrier aggregation (TC 9.1.6.1 and 9.1.6.2) | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123920 | 0613 | - | Uncertainties and Test Tolerances for E-UTRAN FDD, TDD Inter frequency reselection in the existence of non-allowed CSG cell Test cases 4.2.7 and 4.2.8 | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123935 | 0614 | - | Addition of new TC 5.3.6 E-UTRAN TDD-cdma2000 1X Handover | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123936 | 0615 | - | Addition of new TC 6.3.7 E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123937 | 0616 | - | Addition of new TC 6.3.12 E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123938 | 0617 | - | Addition of new TC 9.2.4A.1 FDD - TDD Inter Frequency Absolute RSRQ Accuracy | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123939 | 0618 | - | Addition of new TC 9.2.4A.2 FDD - TDD Inter Frequency Relative Accuracy of RSRQ | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123940 | 0619 | - | Addition of new TC 8.14.3 E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123941 | 0620 | - | Addition of new TC 8.15.3 E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123959 | 0621 | - | Introduction of E-UTRAN FDD-FDD Event Triggered reporting on deactivated SCell with PCell interruption in non-DRX for CA | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123960 | 0622 | - | Introduction of E-UTRAN Event Triggered reporting on deactivated SCell with PCell interruption in non-DRX for CA in Annex | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123995 | 0623 | - | Addition of new TC 8.14.2 E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123996 | 0624 | - | Addition of new TC 8.15.2 E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123998 | 0625 | - | RRM: Further avoidance of frequency overlapping for inter-frequency and inter-RAT | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123999 | 0626 | - | Adding clauses in 3GPP TS36.521-3 with references for positioning test cases | 10.1.0 | 10.2.0 |
| 2012-09 | RAN#57 | R5-123789 | 0605 | - | Implementation of only partly implemented CR: Clarification of the release of UTRAN-EUTRAN Inter-RAT RRM test cases in 36.521-3 | 10.2.0 | 10.2.1 |
| 2012-12 | RAN#58 | R5-124155 | 0667 | - | New TC(8.20) introduction of inter-frequency/RAT measurements in CA mode | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-124172 | 0668 | - | Correction to TC 8.16.3 E-UTRAN FDD-FDD Event Triggered reporting on deactivated SCell with PCell interruption in non-DRX for CA | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-124173 | 0669 | - | Corrections to TC 8.16.4 E-UTRAN TDD-TDD Event Triggered reporting on deactivated SCell with PCell interruption in non-DRX for CA | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-124174 | 0670 | - | Introduction of default RRC messages exceptions for Carrier Aggregation | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-124180 | 0671 | - | Addition of new cell configuration mapping for CA related test cases | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125209 | 0627 | - | RRM TC 9.1.6.1: General updates and corrections | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125210 | 0628 | - | RRM TC 9.1.6.2: General updates and corrections | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125328 | 0629 | - | RRM Annex C: Addition of physical channel settings for eICIC tests | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125335 | 0630 | - | RRM Annex A: Addition of measurement channels for eICIC tests | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125337 | 0631 | - | RRM New TC 7.3.9: Test skeleton for E-UTRAN FDD Radio Link Monitoring Out-of-sync with non-MBSFN ABS | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125338 | 0632 | - | RRM New TC 7.3.10: Test skeleton for E-UTRAN TDD Radio Link Monitoring Out-of-sync with non-MBSFN ABS | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125339 | 0633 | - | RRM New TC 7.3.11: Test skeleton for E-UTRAN FDD Radio Link Monitoring In-sync with non-MBSFN ABS | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125340 | 0634 | - | RRM New TC 7.3.12: Test skeleton for E-UTRAN TDD Radio Link Monitoring In-sync with non-MBSFN ABS | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125341 | 0635 | - | RRM New TC 8.1.7: Test skeleton for E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting with non-MBSFN ABS | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125342 | 0636 | - | RRM New TC 8.2.5: Test skeleton for E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting with non-MBSFN ABS | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125344 | 0637 | - | RRM TC 9.6.2: General corrections | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125363 | 0638 | - | Update Test cases 5.2.3+5.2.6 uncertainties for >3GHz | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125364 | 0639 | - | Fading margin for RRM Test cases 8.3.1, 8.3.2, 8.4.1, 8.4.2, 8.14.1 and 8.15.1 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125366 | 0640 | - | Procedure and requirements for Test cases 4.2.7 and 4.2.8 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125368 | 0641 | - | Correction of RSRP values in the Test Requirement for RRM Test case 5.3.1 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125382 | 0642 | - | Clean up of TDD related RRM tests in 36.521-3 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125427 | 0643 | - | Correction to TC 9.2.5.1 FDD Absolute RSRQ for E-UTRA Carrier Aggregation tests | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125430 | 0644 | - | Correction to TC 9.2.6.1 TDD Absolute RSRQ for E-UTRA Carrier Aggregation tests | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125538 | 0645 | - | RRM TC 9.6.1: Introduction of GSM RSSI accuracy for E-UTRAN FDD | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125549 | 0646 | - | Uncertainties and Test Tolerances for TC 4.5.2.1 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125551 | 0647 | - | Uncertainties and Test Tolerances for TC 5.3.5 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125553 | 0648 | - | Uncertainties and Test Tolerances for TC 8.14.2 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125555 | 0649 | - | Uncertainties and Test Tolerances for TC 8.15.2 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125557 | 0650 | - | Uncertainties and Test Tolerances for TC 8.14.3 and 8.15.3 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125559 | 0651 | - | Uncertainties and Test Tolerances for TC 9.2.4A.1 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125561 | 0652 | - | Uncertainties and Test Tolerances for TC 9.2.4A.2 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125576 | 0665 | - | Correction to Table 9.2.1.1.5-2 in subclause 9.2.1.1.5 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125812 | 0653 | - | Correction to RRM 9.3.1 in 36.521-3 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125813 | 0654 | - | Addition of new TC 8.16.1 E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX including uncertainties and Test Tolerances | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125814 | 0655 | - | Addition of new TC 8.16.2 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX including uncertainties and Test Tolerances | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125865 | 0656 | - | Introduction of Band 27 to TS 36.521-3 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125878 | 0657 | - | Correction to TC 9.2.5.2 FDD Relative RSRQ for E-UTRA Carrier Aggregation tests | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125879 | 0658 | - | Correction to TC 9.2.6.2 TDD Relative RSRQ for E-UTRA Carrier Aggregation tests | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125888 | 0659 | - | Correction to accuracy requirements in RSRP/RSRQ test for Band 26in 36.521-3 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125889 | 0660 | - | Correction to RRM 9.4.1 in 36.521-3 | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-125922 | 0661 | - | Update Test Procedure and Test Tolerances for UE Transmit Timing Accuracy | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-126045 | 0663 | - | Addition of a new TC 9.1.7.1 TDD Absolute RSRP Accuracy E-UTRA for Carrier Aggregation | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-126046 | 0664 | - | Addition of a new TC 9.1.7.2 TDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation | 10.3.0 | 10.4.0 |
| 2012-12 | RAN#58 | R5-126064 | 0662 | - | Correction to RRM 4.2.7 in 36.521-3 | 10.3.0 | 10.4.0 |
| 2013-03 | RAN#59 | R5-130058 | 0666 | - | Uncertainties and Test Tolerances for RRM test cases 9.1.6.1 and 9.1.7.1 | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130060 | 0667 | - | Uncertainties and Test Tolerances for RRM test cases 9.1.6.2 and 9.1.7.2 | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130062 | 0668 | - | Uncertainties and Test Tolerances for RRM test cases 9.2.5.1 and 9.2.6.1 | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130162 | 0705 | - | Modifying test requirements for handover test from E-UTRAN to UTRAN TDD | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130290 | 0669 | - | RRM:Corrections to TC 9.3.1 | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130300 | 0670 | - | RRM: Corrections to TC 9.6.1 | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130301 | 0671 | - | RRM: Corrections to TC 9.6.2 | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130302 | 0672 | - | Editors note for test cases where Test Requirement not valid above 3GHz | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130395 | 0673 | - | Uncertainties and Test Tolerances for TC 6.3.1 Redirection from E-UTRAN FDD to UTRAN FDD | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130396 | 0674 | - | Uncertainties and Test Tolerances for TC 6.3.3 Redirection from E-UTRAN FDD to GERAN when System Information is provided | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130397 | 0675 | - | Uncertainties and Test Tolerances for TC 6.3.9 Redirection from E-UTRAN FDD to UTRAN FDD without System Information | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130398 | 0676 | - | Uncertainties and Test Tolerances for TC 6.3.10 Redirection from E-UTRAN FDD to GERAN when System Information is not provided | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130430 | 0677 | - | Correction to RRM 4.2.7 and 4.2.8 | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130436 | 0678 | - | Update of cell configuration mapping in Annex E | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130468 | 0706 | - | Correction to RRM measurement accuracy tests | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130803 | 0680 | - | Updates to TC 8.16.3 and 8.16.4 | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130931 | 0683 | - | Update Test Procedure and Test Tolerances for UE Transmit Timing Accuracy | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130932 | 0684 | - | Uncertainties and Test Tolerances for RRM test cases 9.2.5.2 and 9.2.6.2 | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130934 | 0685 | - | Test Tolerances to TCs 8.16.3 and 8.16.4 Event Triggered reporting on deactivated SCell with PCell interruption in non-DRX | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130781 | 0686 | - | RRM: Updates and corrections to TC 8.1.5 | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130943 | 0687 | - | Correction to sr-ConfigIndex in RRM FDD-TDD dual mode tests | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130953 | 0688 | - | CA RRM: Corrections to tests with independent events | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130962 | 0689 | - | Add structure for new eICIC test cases 9.2.7.1 and 9.2.8.1, FDD and TDD RSRQ | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130981 | 0690 | - | Introduction of Chapter 9 RRM test cases for RSRP accuracy for FDD EUTRA -eICIC | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130982 | 0691 | - | Introduction of Chapter 9 RRM test cases for RSRP accuracy for TDD EUTRA -eICIC | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130983 | 0692 | - | Additions to TC 8.16.3 E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130984 | 0693 | - | Additions to TC 8.16.4 E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130985 | 0694 | - | Additions to TC 9.2.5.1 FDD Absolute RSRQ for E-UTRA Carrier Aggregation tests | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130986 | 0695 | - | Additions to TC 9.2.5.2 FDD Relative RSRQ for E-UTRA Carrier Aggregation tests | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130987 | 0696 | - | Additions to TC 9.2.6.1 TDD Absolute RSRQ for E-UTRA Carrier Aggregation tests | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130988 | 0697 | - | Additions to TC 9.2.6.2 TDD Relative RSRQ for E-UTRA Carrier Aggregation tests | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130989 | 0698 | - | Updates to TC 8.16.1 and 8.16.2 | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130990 | 0699 | - | Updates to TCs in clause 8.20 | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130991 | 0700 | - | Updates to TCs about FDD and TDD RSRP accuracy for E-UTRA CA | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130992 | 0701 | - | Updates to TCs about FDD and TDD RSRQ accuracy for E-UTRA CA | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130993 | 0702 | - | Updates to H.4 default RRC message content for CA | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130995 | 0703 | - | Correction to RRC Connection Release with Redirection tests | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130782 | 0704 | - | Correction to new CGI E-UTRA cell with autonomous gaps tests | 10.4.0 | 10.5.0 |
| 2013-03 | RAN#59 | R5-130452 | 0679 | - | Adding operating bands 28 and 44 to Annex I in 3GPP TS36.521-3 | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | R5-130906 | 0681 | - | Introduction of Band 44 for APAC 700 MHz | 10.5.0 | 11.0.0 |
| 2013-03 | RAN#59 | R5-130907 | 0682 | - | Introduction of Band 28 for APAC 700 MHz | 10.5.0 | 11.0.0 |
| 2013-06 | RAN#60 | R5-131105 | 0707 | - | Correction to Test requirement for tests 7.1.X | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131109 | 0708 | - | eICIC RRM: Addition of MBSFN ABS configuration in annex | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131110 | 0709 | - | eICIC RRM: Further specification of TC 7.3.9 | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131111 | 0710 | - | eICIC RRM: Further specification of TC 7.3.10 | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131112 | 0711 | - | eICIC RRM: Further specification of TC 7.3.11 | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131113 | 0712 | - | eICIC RRM: Further specification of TC 7.3.12 | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131118 | 0713 | - | CA RRM: References to connection diagrams | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131148 | 0714 | - | Updates to RRM test case for CA band combo CA_2A-29A | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131177 | 0715 | - | RRM: FGI bit support in test applicability statement | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131178 | 0716 | - | RRM TC 9.6: Clarification on testing requirement when no BCCH1 report available | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131180 | 0718 | - | RRM: Uncertainties and test tolerances for TCs 8.1.5 and 8.1.6 | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131182 | 0719 | - | RRM: Uncertainties and test tolerances for TCs 8.3.4 and 8.3.5 | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131185 | 0720 | - | RRM TC 8.14.2: Minor correction to cross references | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131186 | 0721 | - | RRM: Several corrections to CGI related test cases | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131282 | 0722 | - | Uncertainties and Test Tools for eICIC RRM test cases 9.2.7.1, 9.2.8.1 | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131286 | 0723 | - | Uncertainties and Test Tools for RRM test cases 8.11.5, 8.11.6 | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131288 | 0724 | - | Uncertainties and Test Tools for RRM test cases 9.5.1, 9.5.2 | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131291 | 0725 | - | Cleanup of Annex F Introductory Text | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131403 | 0726 | - | Uncertainties and Test Tolerances for TS 36.521-3 test cases 5.2.4 and 5.2.5 | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131404 | 0727 | - | Uncertainties and Test Tolerances for TS 36.521-3 test case 8.5.4 | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131405 | 0728 | - | Uncertainties and Test Tolerances for TS 36.521-3 test cases 8.7.4 and 8.9.2 | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131445 | 0729 | - | Addition of inter-freq/RAT without measurement gaps TCs | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131463 | 0730 | - | Uncertainties and Test Tolerances for RRM test cases 6.3.2 and 6.3.12 | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131468 | 0731 | - | Uncertainties and Test Tolerances for RRM test cases 6.3.5+6.3.6+6.3.7+6.3.8 | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131469 | 0732 | - | Modification to test cases 8.7.1 and 8.7.2 | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131517 | 0733 | - | Corrections to RRM requirements for interruption in single CA | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131529 | 0734 | - | Corrections for RSRQ E-UTRA CA | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131586 | 0735 | - | Correction to the test cases with the existence of non-allowed CSG cell | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131590 | 0736 | - | Correction to PRACH configuration | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131591 | 0737 | - | Correction to Time Alignment Timer | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131613 | 0738 | - | Editors note for test cases where Test Requirement not valid above 3GHz, clause 8 | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131615 | 0739 | - | Editors note for test cases where Test Requirement not valid above 3GHz, clause 8 - 9 | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131740 | 0740 | - | Corrections of band 26 notes in 3GPP TS36.521-3 | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131913 | 0741 | - | Change the IE value of System Information Block type 19 for TS36.521-3 TDD test case 4.3.4.1. | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131922 | 0742 | - | eICIC RRM: Further specification of TC 8.1.7 | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131923 | 0743 | - | eICIC RRM: Further specification of TC 8.2.5 | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131924 | 0744 | - | Structure for new TCs 9.1.8.1, 9.1.8.2, FDD Absolute and Relative eICIC RSRP, Non-MBSFN | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131925 | 0745 | - | Structure for new TCs 9.1.9.1, 9.1.9.2, TDD Absolute and Relative eICIC RSRP, Non-MBSFN | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131926 | 0746 | - | Procedure and messages for eICIC RRM test cases 9.2.7.1, 9.2.8.1 | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131938 | 0747 | - | Uncertainties and Test Tolerances for RRM test cases 6.3.4 and 6.3.11 | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131940 | 0748 | - | RRM: Uncertainties and test tolerances for TCs 9.6.1 and 9.6.2 | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131965 | 0749 | - | Correction to RRC Connection Release with Redirection tests | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131966 | 0750 | - | Correction to Monitored UTRA cell list size | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131967 | 0751 | - | Correction to UE Transmit Timing Accuracy | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131986 | 0752 | - | Editors note for test cases where Test Requirement not valid above 3GHz, clauses 4 to 7 | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-131992 | 0753 | - | Addition of Chapter 9 RRM test cases for absolute RSRQ accuracy under Time-Domain Measurement Resource Restriction with MBSFN ABS | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-132075 | 0754 | - | Correction to new CGI E-UTRA cell with autonomous gaps tests | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-132107 | 0755 | - | Update of Annex E | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-132112 | 0756 | - | Correction to Combined E-UTRAN - E-UTRA and GSM cell search | 11.0.0 | 11.1.0 |
| 2013-06 | RAN#60 | R5-132114 | 0757 | - | Addition of Band 27 to overlooked clauses of TS 36.521-3 | 11.0.0 | 11.1.0 |
| 2013-09 | RAN#61 | R5-133101 | 0758 | - | Uncertainties and Test Tolerances for eICIC Absolute RSRP test cases 9.1.8.1+9.1.9.1 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133103 | 0759 | - | Uncertainties and Test Tolerances for eICIC Relative RSRP test cases 9.1.8.2+9.1.9.2 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133105 | 0760 | - | Uncertainties and Test Tolerances update for Inter-freq RSRP Test cases 9.1.3.x and 9.1.4.x | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133142 | 0761 | - | Addition of test cases 7.3.13 and 7.3.15 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133220 | 0762 | - | RRM: Uncertainties and test tolerances for TCs 9.6.1 and 9.6.2 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133351 | 0764 | - | Uncertainties and Test Tolerances update for Intra-freq RSRP Test cases 9.1.1.1 and 9.1.2.1 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133354 | 0765 | - | Band 28 update for Intra-freq relative RSRP Test case 9.1.1.2 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133421 | 0766 | - | Correction to Common Exception messages for ReportConfig-A6 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133430 | 0767 | - | Correction to 8.7.3 of 36.521-3 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133432 | 0768 | - | Correction to 8.11.5 and 8.11.6 of 36.521-3 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133433 | 0769 | - | Correction to 9.5.1 and 9.5.2 of 36.521-3 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133435 | 0770 | - | Correction to Common Exception messages in RLM tests | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133440 | 0771 | - | Addition of TC 7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133441 | 0772 | - | Addition of TC 7.3.16 E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133447 | 0793 | - | Correction of configurations in RSRP TDD absolute and relative accuracy for CA test cases | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133528 | 0773 | - | Corrections to Conditions for UE Measurements Procedures in RRC_CONNECTED State | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133722 | 0763 | - | eICIC RRM: Message contents for TC-s 7.3.9-12, 8.1.7 and 8.2.5 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133727 | 0788 | - | Corrections to RRM CA measurement accuracy CA test cases with PCell and SCell switching | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133728 | 0789 | - | Correction to E-UTRAN RSRP and RSRQ accuracy for CA test cases of 36.521-3 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133729 | 0791 | - | Correction to CGI test cases | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133809 | 0774 | - | Correction to RRM CA test case 8.16.1 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133810 | 0775 | - | Correction to RRM CA test case 8.16.3 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133813 | 0776 | - | Correction to 8.11.4 InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133814 | 0777 | - | Addition of Uplink-downlink configuration in 6.3.5 of 36.521-3 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133836 | 0778 | - | Cell Timing offsets and Cell Timing uncertainties for CA 8.x and 9.x test cases | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133842 | 0779 | - | Correction to 9.6.1 and 9.6.2 of 36.521-3 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133855 | 0780 | - | Addition of new CA TC 8.16.5 E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133856 | 0781 | - | Addition of new CA TC 8.16.6 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133857 | 0782 | - | Addition of new CA TC 9.2.12.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133858 | 0783 | - | Addition of new CA TC 9.2.12.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133859 | 0784 | - | Correction to RRM CA test case 8.20.1 | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133860 | 0785 | - | New RRM TC 9.1.13.2 TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133861 | 0786 | - | New RRM TC 9.1.13.1 TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133862 | 0787 | - | Cell configuration mapping for new CA TCs for 20MHz | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133896 | 0790 | - | Addition of RMC and OCNG pattern for CA 20 MHz | 11.1.0 | 11.2.0 |
| 2013-09 | RAN#61 | R5-133901 | 0792 | - | CA RRM: Clarification of applicability and completeness status of CA RRM TC-s | 11.1.0 | 11.2.0 |
| 2013-12 | RAN#62 | R5-134145 | 0794 | - | CA RRM: Uncertainties and test tolerances for TC 9.1.13.1 | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134147 | 0795 | - | CA RRM: Uncertainties and test tolerances for TC 9.1.13.2 | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134149 | 0796 | - | CA RRM: Uncertainties and test tolerances for TC 9.2.12.1 | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134151 | 0797 | - | CA RRM: Uncertainties and test tolerances for TC 9.2.12.2 | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134168 | 0798 | - | Clarification of CGI minimum conformance requirements | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134186 | 0799 | - | Introduction of TC 8.1.8 E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting (feICIC) | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134187 | 0800 | - | Introduction of TC 8.2.6 E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting (feICIC) | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134188 | 0801 | - | Introduction 9.2.13 FDD RSRQ under Time Domain Measurement Resource Restriction (feICIC) | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134189 | 0802 | - | Introduction 9.2.14 TDD RSRQ under Time Domain Measurement Resource Restriction (feICIC) | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134197 | 0803 | - | Receiver sensitivity relaxation of UE for High and Low operating band | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134221 | 0808 | - | Uncertainties and Test Tolerances for E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions test case 8.20.3 | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134223 | 0809 | - | Uncertainties and Test Tolerances for eICIC RSRQ test cases 9.2.9.1 and 9.2.10.1 | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134246 | 0810 | - | Reported value correction for RRM test case 9.1.7.1 | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134319 | 0811 | - | RRM: Correction to FGI bit support for TCs 9.1.5.1 and 9.1.5.2 | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134483 | 0813 | - | Update of cell configuration mapping for eICIC RRM in Annex E | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134491 | 0814 | - | Correction to InterRat HO test case with UTRAN TDD | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134492 | 0815 | - | Correction to Cell time offset in TDD Inter-RAT test cases | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134813 | 0820 | - | eICIC RRM: Editorial corrections to clause 7 tests | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134814 | 0821 | - | Updates to Chapter 9 RSRP and RSRQ accuracy test cases under Time-Domain Measurement Resource Restriction with MBSFN ABS (eICIC) | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134815 | 0822 | - | Updates to Chapter 7 FDD RLM test cases under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134852 | 0829 | - | Uncertainties and Test Tolerances update for Intra-freq RSRQ Test cases 9.2.1.1 and 9.2.2.1 | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134862 | 0830 | - | RRM: Updates to the reference measurement channels and OCNG patterns | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134863 | 0831 | - | Introduction of TDD configuration definitions | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134864 | 0832 | - | Update of RRM Requirements Exceptions | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134872 | 0833 | - | Clarification of RRM tests for multiple bandwidths | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134873 | 0834 | - | Addition of new TC E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134874 | 0835 | - | Update of cell configuration mapping for CA in Annex E | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134990 | 0837 | - | CA RRM: Corrections to CA event triggered tests on deactivated SCell with PCell interruption in non-DRX | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-135066 | 0853 | - | CA RRM: Corrections to the message contents exceptions for some CA 20 MHz test cases | 11.2.0 | 11.3.0 |
| 2013-12 | RAN#62 | R5-134216 | 0805 | - | Introduction of FDD Inter frequency absolute RSRP for 5MHz Bandwidth Test case 9.1.17.1 | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-134218 | 0807 | - | Introduction of FDD Intra frequency absolute RSRQ for 5MHz Bandwidth Test case 9.2.17.1 | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-134579 | 0816 | - | Introduction of Band 31 to Annex I conditions for RRM measurements | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-134583 | 0817 | - | Addition of new TC 5.2.11 E-UTRAN FDD - UTRAN FDD handover for 5MHz Bandwidth | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-134585 | 0818 | - | Addition of new TC 6.2.5 E-UTRAN FDD - Contention Based Random Access Test for 5MHz Bandwidth | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-134586 | 0819 | - | Addition of new TC 6.2.6 E-UTRAN FDD - Non-Contention Based Random Access Test for 5MHz Bandwidth | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-134819 | 0823 | - | Addition of new TC 9.1.16 FDD Intra frequency RSRP Accuracy for 5MHz Bandwidth | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-134820 | 0824 | - | Addition of new TC 9.1.17.2 FDD Inter frequency RSRP Accuracy for 5MHz Bandwidth | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-134822 | 0826 | - | Addition of new TC 9.2.18 FDD Inter frequency RSRQ Accuracy for 5MHz Bandwidth | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-134823 | 0827 | - | Addition of new TC 9.3.3 E-UTRAN FDD - UTRA FDD CPICH RSCP absolute accuracy for 5MHz bandwidth | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-134824 | 0828 | - | Addition of new TC 9.4.3 E-UTRAN FDD - UTRA FDD CPICH Ec/No absolute accuracy for 5MHz bandwidth | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-134898 | 0836 | - | Update of band configuration table for Inter Band | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-135033 | 0838 | - | Addition of new TC 4.2.9 E-UTRAN FDD-FDD intra-frequency Cell Re-selection case for 5MHz | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-135034 | 0839 | - | Addition of new TC 4.3.1.4 E-UTRAN FDD - UTRAN FDD cell re-selection: UTRA FDD is of lower priority for 5MHz | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-135035 | 0840 | - | Addition of new TC 5.1.9 E-UTRAN FDD-FDD Intra frequency handover for 5MHz | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-135036 | 0841 | - | Addition of new TC 6.1.5 E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz Bandwidth | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-135037 | 0842 | - | Addition of new TC 7.1.5 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for 5MHz Bandwidth | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-135038 | 0843 | - | Addition of new TC 7.2.3 E-UTRAN FDD - UE Timing Advance Adjustment Accuracy Test for 5MHz Bandwidth | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-135039 | 0844 | - | Addition of new TC 7.3.23 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-135040 | 0845 | - | Addition of new TC 7.3.24 E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-135041 | 0846 | - | Addition of new TC 7.3.25 E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-135042 | 0847 | - | Addition of new TC 8.1.9 E-UTRAN FDD-FDD intra frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-135043 | 0848 | - | Addition of new TC 8.1.10 E-UTRAN FDD-FDD intra frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for 5MHz bandwidth | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-135044 | 0849 | - | Addition of new TC 8.5.7 E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions for 5MHz bandwidth | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-135047 | 0850 | - | Updates to TCs 4.3.1.2 and 4.3.3 | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-135048 | 0851 | - | Updates to TCs 5.2.1 and 5.2.2 | 11.3.0 | 12.0.0 |
| 2013-12 | RAN#62 | R5-135049 | 0852 | - | Updates the test system uncertainties and test tolerance for TC 6.2.1, 6.2.2, 6.2.3 and 6.2.4 | 11.3.0 | 12.0.0 |
| 2014-03 | RAN#63 | R5-140085 | 0854 | - | CA RRM: Uncertainties and test tolerances for TC 8.20.1 and 8.20.2 | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140088 | 0855 | - | Corrections to UE Transmit Timing minimum requirements | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140089 | 0856 | - | Corrections to measurements of the SCC with deactivated SCell | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140090 | 0857 | - | Corrections to TC 8.16.3 E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140234 | 0859 | - | Uncertainties and Test Tolerances for FDD Intra frequency relative RSRP for 5MHz Bandwidth Test case 9.1.16.2 and update 9.1.1.2, 9.1.2.2. | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140236 | 0860 | - | Uncertainties and Test Tolerances for FDD Inter Frequency Absolute RSRQ Accuracy for 5MHz Bandwidth 9.2.18.1, and update Test cases 9.2.3.1, 9.2.4.1. | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140238 | 0861 | - | Uncertainties and Test Tolerances for FDD Inter Frequency Relative RSRQ Accuracy for 5MHz Bandwidth 9.2.18.2, and update Test cases 9.2.3.2, 9.2.4.2. | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140240 | 0862 | - | Uncertainties and Test Tolerances for E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions test case 8.20.4 | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140241 | 0863 | - | Noc uncertainty for Test Cases 8.5.1, 8.5.3, 8.5.7 and 8.6.1 | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140255 | 0864 | - | CA RRM: Corrections to event triggered reporting under deactivated SCell in non-DRX tests | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140259 | 0865 | - | CA RRM: Correction and clarification of messages exception on the maximum number of cells to be measured | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140289 | 0867 | - | Uncertainties and Test Tolerances for RRM test case 7.3.24 | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140295 | 0868 | - | Uncertainties and Test Tolerances for RRM test case 9.3.3 | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140297 | 0869 | - | Uncertainties and Test Tolerances for RRM test case 9.4.3 | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140301 | 0870 | - | Addition of cell configuration mapping for LTE 450 related test cases | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140401 | 0871 | - | Introduction of operating band groups to TS36.521-3, clause 3 | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140404 | 0872 | - | Introduction of operating band groups to TS36.521-3, Annex I | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140423 | 0873 | - | Correction to test applicability of measurement without gaps | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140483 | 0874 | - | Completion and alignment of 5MHz Bandwidth Test cases 9.1.17.1, 9.2.17.1 | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140501 | 0875 | - | Update of exceptions in eICIC RRM with Non-MBSFN | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140502 | 0876 | - | Update of eICIC RLM test cases | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140512 | 0877 | - | Correction of UTRA TDD Redirection tests | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140513 | 0878 | - | Correction to CA RRM Inter frequency tests | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140562 | 0879 | - | Correction to GSM RSSI absolute accuracy tests | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140597 | 0880 | - | TC 9.3.1, 9.4.1 Corrections to test procedures in 3GPP TS36.521-3 | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140810 | 0881 | - | Correction of b2-threshold for EUTRA and GERAN cell | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140849 | 0883 | - | Additions to TC 8.1.8 E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting (feICIC) | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140850 | 0884 | - | Additions to TC 8.2.6 E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting (feICIC) | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140851 | 0885 | - | Additions to TC 9.2.13 FDD RSRQ under Time Domain Measurement Resource Restriction (feICIC) | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140852 | 0886 | - | Additions to TC 9.2.14 TDD RSRQ under Time Domain Measurement Resource Restriction (feICIC) | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140853 | 0887 | - | Addition to TC 7.3.21 FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and MBSFN ABS (feICIC) | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140854 | 0888 | - | Addition to TC 7.3.22 TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and MBSFN ABS (feICIC) | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140855 | 0889 | - | Addition to TC 9.1.15.2 TDD Intra Frequency Relative RSRP Accuracy under TD Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140863 | 0890 | - | Corrections to minimum requirements for time domain measurement | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140865 | 0891 | - | Uncertainties and Test Tolerances for RRM test case 9.1.16.1 | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140866 | 0892 | - | Uncertainties and Test Tolerances for RRM test case 9.1.17.2 | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140869 | 0893 | - | Corrections to test cases 8.16.3 and 8.16.4 | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140876 | 0895 | - | Introduction of feICIC to Annex H | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140882 | 0896 | - | Addition to TC 7.3.19 FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and Non-MBSFN ABS (feICIC) | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140883 | 0897 | - | Addition to TC 7.3.20 TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and Non-MBSFN ABS (feICIC) | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140884 | 0898 | - | Addition to TC 9.1.14.1 FDD Intra Frequency Absolute RSRP Accuracy under TD Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140885 | 0899 | - | Addition to TC 9.1.14.2 FDD Intra Frequency Relative RSRP Accuracy under TD Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-140886 | 0900 | - | Addition to TC 9.1.15.1 TDD Intra Frequency Absolute RSRP Accuracy under TD Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-141021 | 0901 | - | Introduction of operating band groups to TS36.521-3, test cases | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-141022 | 0902 | - | Addition of WB-RSRQ measurement test cases | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-141023 | 0903 | - | Update of intra-band CA cell mapping in Annex E | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-141024 | 0904 | - | Clarification on operating band configuration for inter-RAT RRM test cases | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-141031 | 0905 | - | update to TC 9.4.1 L2W CPICH Ec/No Absolute accuracy | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-141036 | 0906 | - | Addition of new CA RRM TC 7.1.3 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for SCell | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-141037 | 0907 | - | Addition of new CA RRM TC 7.1.4 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-141049 | 0908 | - | Correction to Test2 of UE Transmit Timing Accuracy test cases 7.1.1 and 7.1.2 | 12.0.0 | 12.1.0 |
| 2014-03 | RAN#63 | R5-141050 | 0866 | - | Updates Test cases about UE Transmit Timing Accuracy Tests | 12.0.0 | 12.1.0 |
| 2014-06 | RAN#64 | R5-142167 | 0909 | - | Correction of clause 9.2.7.1 in 36.521-3 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142180 | 0910 | - | Correction on PDSCH allocation in PRS subframe | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142212 | 0911 | - | Additions to feICIC RRM test cases in Annex F | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142222 | 0912 | - | Cleanup of operating band groups in 3GPP TS36.521-3 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142547 | 0913 | - | Uncertainties and Test Tolerances for eICIC Intra-Frequency Event-Triggered Reporting test cases 8.1.7 and 8.2.5 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142550 | 0914 | - | Uncertainties and Test Tolerances for eICIC RLM out-of-sync Test cases 7.3.9,7.3.10,7.3.13 and 7.3.14 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142553 | 0915 | - | Uncertainties and Test Tolerances for eICIC RLM In-sync Test cases 7.3.11,7.3.12,7.3.15 and 7.3.16 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142600 | 0916 | - | Clean-up of eICIC RRM tests | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142621 | 0918 | - | Clean-up for E-UTRAN RRM test cases | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-142623 | 0919 | - | Update of Annex E for CA RRM Inter frequency tests for Intra-band | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143018 | 0920 | - | Correction of clause 9.2.8.1 in 36.521-3 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143019 | 0921 | - | Update eICIC Absolute RSRP Test cases 9.1.8.1+9.1.9.1 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143020 | 0922 | - | Update eICIC Absolute RSRQ Test cases 9.2.7.1+9.2.8.1 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143022 | 0923 | - | Uncertainties and test tolerance for RRM Test case 9.1.10.1, 9.1.11.1 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143023 | 0924 | - | Uncertainties and test tolerance for RRM Test case 9.1.10.2, 9.1.11.2 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143024 | 0925 | - | Update to eICIC Absolute RSRQ with MBSFN ABS for chapter 9 RRM test cases | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143057 | 0926 | - | Updates to CA RRM 8.16.x test cases | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143058 | 0927 | - | Correction to UL timing accuracy Tx test cases | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143059 | 0928 | - | Update of Annex E for InterRAT with TDD-TD-SCDMA | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143062 | 0929 | - | Additions to TC 8.2.6 E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting (feICIC) | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143063 | 0930 | - | Update eICIC Relative RSRP Test cases 9.1.8.2+9.1.9.2 | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143064 | 0931 | - | Additions to TC 9.2.13 FDD RSRQ under Time Domain Measurement Resource Restriction (feICIC) | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143065 | 0932 | - | Additions to TC 9.2.14 TDD RSRQ under Time Domain Measurement Resource Restriction (feICIC) | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143066 | 0933 | - | Addition of RLM test cases with non-MBSFN for fEICIC | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143067 | 0934 | - | Test Cases 9.1.14.1, 9.1.14.2, 9.1.15.1, 9.1.15.2 for feICIC | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143085 | 0935 | - | Addition of new TC 8.16.10 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143086 | 0936 | - | Addition of new TC 9.1.18 FDD RSRP Accuracy for E-UTRAN Carrier Aggregation for 10MHz + 5MHz | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143087 | 0937 | - | Addition of new TC 9.1.19 TDD RSRP Accuracy for E-UTRAN Carrier Aggregation for 10MHz + 5MHz | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143088 | 0938 | - | Addition of new TC 9.2.21 FDD RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143089 | 0939 | - | Addition of new TC 9.2.22 TDD RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143090 | 0940 | - | Addition of new TC 8.16.14 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5MHz+5MHz | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143091 | 0941 | - | Addition of new TC 9.1.20 FDD RSRP Accuracy for E-UTRAN Carrier Aggregation for 5MHz + 5MHz | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143092 | 0942 | - | Addition of new TC 9.1.21 TDD RSRP Accuracy for E-UTRAN Carrier Aggregation for 5MHz + 5MHz | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143093 | 0943 | - | Addition of new TC 9.2.23 FDD RSRQ Accuracy for E-UTRAN Carrier Aggregation for 5MHz + 5MHz | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143094 | 0944 | - | Addition of new TC 9.2.24 TDD RSRQ Accuracy for E-UTRAN Carrier Aggregation for 5MHz + 5MHz | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143102 | 0945 | - | Additions to TC 8.1.8 E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting (feICIC) | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143103 | 0946 | - | Additions to feICIC to Annex H | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143108 | 0947 | - | Correction to periodicity of ABS pattern in eICIC RRM | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143112 | 0948 | - | Additions to TDD interruption requirements for SCell | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143152 | 0949 | - | Correction to event triggered reporting tests for CA | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143153 | 0950 | - | Correction to Intra CA test cases | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143157 | 0951 | - | TC 7.3.19 FDD Radio Link Monitoring for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and Non-MBSFN ABS (feICIC) | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143158 | 0952 | - | TC 7.3.20 TDD Radio Link Monitoring for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and Non-MBSFN ABS (feICIC) | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143159 | 0953 | - | TC 7.3.21 FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and MBSFN ABS (feICIC) | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143160 | 0954 | - | TC 7.3.22 TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and MBSFN ABS (feICIC) | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143172 | 0955 | - | Addition of new TC 8.16.9 E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143173 | 0956 | - | Addition of new TC 8.16.13 E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz+5MHz | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143176 | 0957 | - | Addition of new TC 8.16.7 E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143191 | 0958 | - | Clarification of tracking area updating procedure | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | R5-143209 | 0917 | 1 | Correction to 9.6.2 GSM RSSI accuracy for E-UTRAN TDD | 12.1.0 | 12.2.0 |
| 2014-06 | RAN#64 | - | - | - | Correction of misplaced Table F.3.2-1 entry of R5-142547 | 12.2.0 | 12.2.1 |
| 2014-09 | RAN#65 | R5-144124 | 0959 | - | Corrections to RRM Physical Cell Id (PCI) Configuration Conditions in feICIC | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144169 | 0962 | - | RRM: Corrections to TCs 9.2.18.x (5MHz) | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144170 | 0963 | - | eICIC RRM: Corrections to TCs 7.3.14 and 7.3.16 | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144171 | 0964 | - | RRM: Removal of redundant (time limited) editor's notes | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144172 | 0965 | - | CA RRM: Correction of wrong test applicability | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144197 | 0966 | - | Addition of new RRM TC 8.16.11 E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144202 | 0967 | - | Addition of new RRM TC 8.16.16 E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5MHz BW | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144355 | 0973 | - | Correction to Inter-frequency event triggered reporting test in 8.3.6 | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144356 | 0974 | - | Correction to InterRAT event triggered reporting test in 8.5.6 | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144357 | 0975 | - | Update of Annex E | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144361 | 0976 | - | Correction to CQI reporting periodicity in RLM with eICIC | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144362 | 0977 | - | Addition of message exceptions for eICIC test cases with MBSFN | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144767 | 0979 | - | Uncertainties and Test Tolerances for RRM TCs 8.16.7+8.16.8 | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144768 | 0980 | - | Uncertainties and Test Tolerances for RRM TCs 8.16.5+8.16.6 | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144785 | 0988 | - | Uncertainties and test tolerance for 36.521-3 Test case 9.4.1, 9.4.2 and 9.4.3 | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144788 | 0978 | - | Updates to Annex I of TS 36.521-3 | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144810 | 0981 | - | Antenna correlation for RRM Test cases | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144811 | 0982 | - | Correction to RRM event triggered tests | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144812 | 0983 | - | Correction to E-UTRAN CA Measurements | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144830 | 0984 | - | Uncertainties and Test Tolerances for Test case 8.3.6 | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144832 | 0985 | - | Uncertainties and Test Tolerances for Test case 8.5.6 | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144833 | 0986 | - | Uncertainties and Test Tolerances update for Test cases 8.3.3+8.4.3 | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144850 | 0989 | - | Addition of new RRM TC 8.16.12 E-UTRAN TDD Event triggered reporting on deactivating Scells and interruption probability (0.5%) without DRX for 10MHz+5MHz BW | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144851 | 0990 | - | Cell configuration mapping for 5MHz+5MHz and 10MHz+5MHz test cases | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144852 | 0991 | - | Addition of new RRM TC 8.16.15 E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz BW | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144858 | 0992 | - | Update to test procedure for GSM carrier measurement test cases | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144908 | 0993 | - | New TC: FDD Intra Frequency Serving Cell Absolute RSRP Accuracy | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144909 | 0994 | - | New TC: FDD Intra Frequency Serving Cell Absolute RSRQ Accuracy | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144910 | 0995 | - | New TC: TDD Intra Frequency Serving Cell Absolute RSRP Accuracy | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144912 | 0996 | - | Removable of editor's notes for maxReportCells | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144913 | 0997 | - | Correction to neighCellConfig of MeasObjectEUTRA in CGI tests | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144927 | 0998 | - | New TC: TDD Intra Frequency Serving Cell Absolute RSRQ Accuracy | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144124 | 0959 | - | Corrections to RRM Physical Cell Id (PCI) Configuration Conditions in feICIC | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144169 | 0962 | - | RRM: Corrections to TCs 9.2.18.x (5MHz) | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144170 | 0963 | - | eICIC RRM: Corrections to TCs 7.3.14 and 7.3.16 | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144171 | 0964 | - | RRM: Removal of redundant (time limited) editor's notes | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144172 | 0965 | - | CA RRM: Correction of wrong test applicability | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144197 | 0966 | - | Addition of new RRM TC 8.16.11 E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144202 | 0967 | - | Addition of new RRM TC 8.16.16 E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5MHz BW | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144355 | 0973 | - | Correction to Inter-frequency event triggered reporting test in 8.3.6 | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144356 | 0974 | - | Correction to InterRAT event triggered reporting test in 8.5.6 | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144357 | 0975 | - | Update of Annex E | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144361 | 0976 | - | Correction to CQI reporting periodicity in RLM with eICIC | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144362 | 0977 | - | Addition of message exceptions for eICIC test cases with MBSFN | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144767 | 0979 | - | Uncertainties and Test Tolerances for RRM TCs 8.16.7+8.16.8 | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144768 | 0980 | - | Uncertainties and Test Tolerances for RRM TCs 8.16.5+8.16.6 | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144785 | 0988 | - | Uncertainties and test tolerance for 36.521-3 Test case 9.4.1, 9.4.2 and 9.4.3 | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144788 | 0978 | 1 | Updates to Annex I of TS 36.521-3 | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144810 | 0981 | - | Antenna correlation for RRM Test cases | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144811 | 0982 | - | Correction to RRM event triggered tests | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144812 | 0983 | - | Correction to E-UTRAN CA Measurements | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144830 | 0984 | - | Uncertainties and Test Tolerances for Test case 8.3.6 | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144832 | 0985 | - | Uncertainties and Test Tolerances for Test case 8.5.6 | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144833 | 0986 | - | Uncertainties and Test Tolerances update for Test cases 8.3.3+8.4.3 | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144850 | 0989 | - | Addition of new RRM TC 8.16.12 E-UTRAN TDD Event triggered reporting on deactivating Scells and interruption probability (0.5%) without DRX for 10MHz+5MHz BW | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144851 | 0990 | - | Cell configuration mapping for 5MHz+5MHz and 10MHz+5MHz test cases | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144852 | 0991 | - | Addition of new RRM TC 8.16.15 E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz BW | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144858 | 0992 | - | Update to test procedure for GSM carrier measurement test cases | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144908 | 0993 | - | New TC: FDD Intra Frequency Serving Cell Absolute RSRP Accuracy | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144909 | 0994 | - | New TC: FDD Intra Frequency Serving Cell Absolute RSRQ Accuracy | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144910 | 0995 | - | New TC: TDD Intra Frequency Serving Cell Absolute RSRP Accuracy | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144912 | 0996 | - | Removable of editor's notes for maxReportCells | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144913 | 0997 | - | Correction to neighCellConfig of MeasObjectEUTRA in CGI tests | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144927 | 0998 | - | New TC: TDD Intra Frequency Serving Cell Absolute RSRQ Accuracy | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144124 | 0959 | - | Corrections to RRM Physical Cell Id (PCI) Configuration Conditions in feICIC | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144169 | 0962 | - | RRM: Corrections to TCs 9.2.18.x (5MHz) | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144170 | 0963 | - | eICIC RRM: Corrections to TCs 7.3.14 and 7.3.16 | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144171 | 0964 | - | RRM: Removal of redundant (time limited) editor's notes | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144172 | 0965 | - | CA RRM: Correction of wrong test applicability | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144197 | 0966 | - | Addition of new RRM TC 8.16.11 E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144202 | 0967 | - | Addition of new RRM TC 8.16.16 E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5MHz BW | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144355 | 0973 | - | Correction to Inter-frequency event triggered reporting test in 8.3.6 | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144356 | 0974 | - | Correction to InterRAT event triggered reporting test in 8.5.6 | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144357 | 0975 | - | Update of Annex E | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144361 | 0976 | - | Correction to CQI reporting periodicity in RLM with eICIC | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144362 | 0977 | - | Addition of message exceptions for eICIC test cases with MBSFN | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144767 | 0979 | - | Uncertainties and Test Tolerances for RRM TCs 8.16.7+8.16.8 | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144768 | 0980 | - | Uncertainties and Test Tolerances for RRM TCs 8.16.5+8.16.6 | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144785 | 0988 | - | Uncertainties and test tolerance for 36.521-3 Test case 9.4.1, 9.4.2 and 9.4.3 | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144788 | 0978 | 1 | Updates to Annex I of TS 36.521-3 | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144810 | 0981 | - | Antenna correlation for RRM Test cases | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144811 | 0982 | - | Correction to RRM event triggered tests | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144812 | 0983 | - | Correction to E-UTRAN CA Measurements | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144830 | 0984 | - | Uncertainties and Test Tolerances for Test case 8.3.6 | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144832 | 0985 | - | Uncertainties and Test Tolerances for Test case 8.5.6 | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144833 | 0986 | - | Uncertainties and Test Tolerances update for Test cases 8.3.3+8.4.3 | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144850 | 0989 | - | Addition of new RRM TC 8.16.12 E-UTRAN TDD Event triggered reporting on deactivating Scells and interruption probability (0.5%) without DRX for 10MHz+5MHz BW | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144851 | 0990 | - | Cell configuration mapping for 5MHz+5MHz and 10MHz+5MHz test cases | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144852 | 0991 | - | Addition of new RRM TC 8.16.15 E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz BW | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144858 | 0992 | - | Update to test procedure for GSM carrier measurement test cases | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144908 | 0993 | - | New TC: FDD Intra Frequency Serving Cell Absolute RSRP Accuracy | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144909 | 0994 | - | New TC: FDD Intra Frequency Serving Cell Absolute RSRQ Accuracy | 12.2.1 | 12.3.0 |
| 2014-09 | RAN#65 | R5-144910 | 0995 | - | New TC: TDD Intra Frequency Serving Cell Absolute RSRP Accuracy | 12.2.1 | 12.3.0 |
| 2014-12 | RAN#66 | R5-145129 | 1000 | - | CA-RRM: Corrections to TC 8.16.5 and 8.16.6 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145130 | 1001 | - | RRM: Correction to MBSFN subframes configuration | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145245 | 1002 | - | eICIC-RRM: Missing uncertainties for TC 8.1.7 and 8.2.5 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145299 | 1003 | - | Update Uncertainties for Test case 8.5.3 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145301 | 1004 | - | Uncertainties and Test Tolerances for feICIC RSRQ accuracy Test cases | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145303 | 1005 | - | Uncertainties and Test Tolerances for Test cases 9.2.21.1 and 9.2.22.1 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145305 | 1006 | - | Uncertainties and Test Tolerances for Test cases 9.2.21.2 and 9.2.22.2 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145457 | 1007 | - | Addition of new RRM TC 7.1.7 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145464 | 1008 | - | Corrections to RSRP and RSRQ tests | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145477 | 1009 | - | Corrections to TC 7.3.20 E-UTRAN TDD RL Monitoring Test for In-sync under for feICIC | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145488 | 1011 | - | Test Tolerances for TC 8.1.8 E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting (feICIC) | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145489 | 1012 | - | Test Tolerances for TC 8.2.6 E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting (feICIC) | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145493 | 1013 | - | Uncertainties and Test Tolerances to Annex F for feICIC RRM test cases | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145501 | 1014 | - | Correction to periodicity of ABS pattern in RRM for feICIC | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145512 | 1015 | - | Alignment of Annex I | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145528 | 1017 | - | Editorial removal of brackets from TC 7.3.15 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145587 | 1018 | - | Update of feICIC Out-Of-Sync test cases with assistance data | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145639 | 1019 | - | Updates to RRM feICIC test cases in Clause 9 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145801 | 1020 | - | RRM: Modification of test description to refer to State 2A-RF | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145802 | 1021 | - | Update of Annex E cell configuration mapping for feICIC | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145816 | 1022 | - | Updates to RRM TC 7.1.3 and 7.1.4 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145817 | 1023 | - | Correction to the band group in 9.2.1.1 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145819 | 1024 | - | Correction to 8.16.6 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145820 | 1025 | - | Correction to Phys Cell Id of MeasResult in RRM test cases | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145831 | 1026 | - | Uncertainties and Test Tolerances for feICIC Test cases 7.3.17, 7.3.18, 7.3.19, 7.3.20, 7.3.21, 7.3.22 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145832 | 1027 | - | Uncertainties and Test Tolerances for feICIC test cases 9.1.14.1 and 9.1.15.1 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145837 | 1028 | - | Uncertainties and Test Tolerances for feICIC test cases 9.1.14.2 and 9.1.15.2 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145877 | 1029 | - | Addition of new TCs for activation and deactivation of known SCell | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145878 | 1030 | - | Update to test procedure for LTE CA Event reporting test cases | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145906 | 1031 | - | Addition of new RRM TC 6.2.7 E-UTRAN FDD - Non-Contention Based Random Access Test For SCell in sTAG | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145907 | 1032 | - | Addition of new RRM TC 6.2.8 E-UTRAN TDD - Non-Contention Based Random Access Test For SCell in sTAG | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145908 | 1033 | - | Addition of new RRM TC 7.1.6 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for SCell in sTAG | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145909 | 1034 | - | Addition of new RRM TC 7.2.4 E-UTRAN FDD - UE Timing Advance Adjustment Accuracy Test For SCell in sTAG | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145910 | 1035 | - | Addition of new RRM TC 7.2.5 E-UTRAN TDD - UE Timing Advance Adjustment Accuracy Test For SCell in sTAG | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145911 | 1036 | - | Updates to CA TCs about 10+5MHz and 5+5MHz BW | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145912 | 1037 | - | New TCs 9.1.12 for FDD RSRP Accuracy for CA 20MHz | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145913 | 1038 | - | New TCs 9.2.11 for FDD RSRQ Accuracy for CA 20MHz | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145915 | 1039 | - | Correction to measurement performance test cases 9.3.2 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145918 | 1040 | - | Update of Annex H for feICIC test cases | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145919 | 1041 | - | Update of feICIC RLM In-sync test cases with assistance data | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145927 | 1042 | - | Correction to the test procedure in 9.6.1 and 9.6.2 | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145977 | 0999 | - | CA-RRM: Review of test frequencies in Annex E | 12.3.0 | 12.4.0 |
| 2014-12 | RAN#66 | R5-145982 | 1010 | - | Cell mapping for some RRM TCs | 12.3.0 | 12.4.0 |
| 2015-01 | - | - | - | - | Re-implementation of R5-145927 | 12.4.0 | 12.4.1 |
| 2015-01 | - | - | - | - | Re-implementation of R5-145911 | 12.4.0 | 12.4.1 |
| 2015-03 | RAN#67 | R5-150213 | 1043 | - | Corrections to multicarrier handover from E-UTRA to UTRA requirements | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150218 | 1044 | - | Test Tolerances for 4.3.1.3 E-UTRAN FDD - UTRAN FDD cell re-selection | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150220 | 1045 | - | Test Tolerance for 4.3.4.3 EUTRA TDD-UTRA TDD cell reselection | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150222 | 1046 | - | Test Tolerances for 8.5.2 E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150224 | 1047 | - | Test Tolerances for 8.7.3 E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150226 | 1048 | - | Test Tolerances for 8.9.1 E-UTRAN FDD - UTRAN TDD event triggered reporting | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150228 | 1049 | - | Test Tolerances for 8.8.1+8.10.1 E-UTRAN - GSM event triggered reporting | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150230 | 1050 | - | Test Tolerances for 8.8.2+8.10.2 E-UTRAN - GSM event triggered reporting when DRX is used | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150232 | 1051 | - | Uncertainties to Annex F for RRM test cases | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150242 | 1052 | - | Update Uncertainties for Test case 5.1.5 | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150244 | 1053 | - | Update Uncertainties for Test case 5.1.6 | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150246 | 1054 | - | Update Uncertainties for Test case 5.2.3, 5.2.6 | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150256 | 1055 | - | Introduction of Test Cases for RSRQ Accuracy for TDD-FDD CA | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150306 | 1056 | - | Uncerts and Test Tools for TCs 8.16.11+8.16.12 36.521-3 CR | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150310 | 1057 | - | Uncerts and Test Tools for TCs 8.16.15+8.16.16 36.521-3 CR | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150314 | 1058 | - | Uncerts and Test Tools for TCs 9.1.18.2+9.1.19.2 36.521-3 CR | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150316 | 1059 | - | Uncerts and Test Tools for TCs 9.1.20.1+9.1.21.1 36.521-3 CR | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150318 | 1060 | - | Uncerts and Test Tools for TCs 9.1.20.2+9.1.21.2 36.521-3 CR | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150320 | 1061 | - | RRM: Corrections to event triggered tests without measurement gap | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150324 | 1062 | - | Uncerts and Test Tools for TCs 9.2.23.1+9.2.24.1 36.521-3 CR | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150327 | 1063 | - | Uncerts and Test Tools for TCs 9.2.23.2+9.2.24.2 36.521-3 CR | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150328 | 1064 | - | Correction to RRM TCs 9.1.6.2 and 9.1.7.2 | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150361 | 1065 | - | Addition of cell mapping for some test cases | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150381 | 1066 | - | Update of Annex E and F for TCs 9.1.12 and 9.2.11 | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150390 | 1067 | - | Correction to test procedure for LTE CA event reporting test case | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150421 | 1068 | - | Correction to message exceptions in eICIC MBSFN ABS | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150423 | 1069 | - | Correction to feICIC common exceptions in Annex H | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150424 | 1070 | - | Clean up on cl.7 and cl.8 feICIC RRM test cases | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150425 | 1071 | - | Clean up on cl.9 feICIC RRM test cases | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150517 | 1072 | - | Error correction to TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150523 | 1073 | - | Cell configuration mapping for Rel 12 and forward absolute RSRP accuracy RRM cases | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150526 | 1074 | - | Update R8 intra frequency absolute RSRP accuracy requirements | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150527 | 1075 | - | Update R8 inter frequency absolute RSRP accuracy requirements | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150529 | 1076 | - | Update R9 inter frequency absolute RSRP accuracy requirements | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150530 | 1077 | - | Update R10 intra frequency absolute RSRP accuracy requirements | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150531 | 1078 | - | Update TDD RSRP Accuracy requirements for Carrier Aggregation | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150806 | 1079 | - | New TCs: intra frequency absolute RSRP accuracy requirements | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150807 | 1080 | - | New TCs: inter frequency absolute RSRP accuracy requirements | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150808 | 1081 | - | New TCs: RSRP absolute accuracy for E-UTRA CA | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150812 | 1082 | - | New TC: E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150813 | 1083 | - | New TC: E-UTRAN TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20MHz+10MHz bandwidth | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150814 | 1084 | - | New TC: TDD RSRP Accuracy for E-UTRAN Carrier Aggregation for 20MHz + 10MHz | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150815 | 1085 | - | New TC: TDD RSRQ Accuracy for E-UTRAN Carrier Aggregation for 20MHz + 10MHz | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150841 | 1086 | - | Corrections to on time to identify the target UTRA TDD cell for blind redirection | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150842 | 1087 | - | Update of TCs 9.1.12 for FDD RSRP Accuracy for CA 20MHz | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150843 | 1088 | - | Update TCs 9.2.11 for FDD RSRQ Accuracy for CA 20MHz | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150844 | 1089 | - | Correction to eICIC Radio link monitoring test cases | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150845 | 1090 | - | Clean up on cl.7 and cl.9 eICIC RRM test cases | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150847 | 1092 | - | Update R11 RSRP Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150852 | 1093 | - | Uncerts and Test Tools for TCs 8.16.13+8.16.14 36.521-3 CR | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150856 | 1094 | - | Uncerts and Test Tools for TCs 8.16.9+8.16.10 36.521-3 CR | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150857 | 1104 | - | RRM: Inter-frequency testing for Band 30 | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150884 | 1095 | - | Cell configuration mapping for 20MHz +10MHz CA RRM new cases | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150885 | 1096 | - | Editorial correction to 8.14.2 and 8.15.2 | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150886 | 1097 | - | Corrections to RRM test cases in clauses 5 and 8 | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150887 | 1098 | - | Uncerts and Test Tools for TCs 9.1.18.1+9.1.19.1 36.521-3 CR | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150901 | 1099 | - | CA RRM: Correction to test frequencies and Cell-IDs | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150902 | 1100 | - | Addition of CA capability in RRM tests | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150903 | 1101 | - | feICIC RRM: Corrections to Cell-IDs | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150930 | 1102 | - | RRM: Consideration of unsent measurement reports | 12.4.1 | 12.5.0 |
| 2015-03 | RAN#67 | R5-150908 | 1103 | - | Updates of test procedures in activation and deactivation test cases | 12.4.1 | 12.5.0 |
| 2015-06 | RAN#68 | R5-151126 | 1107 | - | RRM: Corrections to message configuration of TCs 9.3.2 and 9.4.2 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151127 | 1108 | - | RRM: Corrections to test procedures of TCs 9.3.x and 9.4.x | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151128 | 1109 | - | CA RRM: Corrections to TC 9.1.24.2 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151133 | 1111 | - | Uncertainties and Test tolerances for Test cases 9.9.1.1 and 9.9.2.1 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151135 | 1112 | - | Uncertainties and Test tolerances for Test cases 9.9.1.2 and 9.9.2.2 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151197 | 1122 | - | Corrections to CPICH Ec/No to CPICH Ec/Io in EUTRA FDD to UTRA FDD HO | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151277 | 1129 | - | Correction to RRM test case 8.2.4 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151278 | 1130 | - | Correction to RRM test case 8.11.4 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151330 | 1150 | - | Addition of band 32 to TS36.521-3 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151464 | 1155 | - | Correction to applicability of TD-LTE to UTRA TDD periodic measurements | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151485 | 1158 | - | Update of eICIC meas pattern for PCell, TC 8.2.5 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151542 | 1162 | - | Uncerts and Test Tools for TC 8.16.21 36.521-3 CR | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151544 | 1163 | - | Uncerts and Test Tools for TC 8.16.22 36.521-3 CR | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151546 | 1164 | - | Uncerts and Test Tools for TCs 9.1.24.1 36.521-3 CR | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151548 | 1165 | - | Uncerts and Test Tools for TCs 9.1.24.1_1 36.521-3 CR | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151550 | 1166 | - | Uncerts and Test Tools for TC 9.1.24.2 36.521-3 CR | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151552 | 1167 | - | Uncerts and Test Tools for TC 9.2.27.1 36.521-3 CR | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151554 | 1168 | - | Uncerts and Test Tools for TC 9.2.27.2 36.521-3 CR | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151830 | 1105 | 1 | RRM: Consideration of unsent measurement reports in recently added test cases | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151831 | 1113 | 1 | Test Tolerances for 8.3.1+8.3.2+8.14.1+8.14.2 inter-frequency event triggered reporting under fading propagation conditions | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151832 | 1115 | 1 | Test Tolerances for 8.4.1+8.4.2+8.15.1+8.15.2 inter-frequency event triggered reporting under fading propagation conditions | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151833 | 1117 | 1 | Test Tolerances for 9.2.4A.1 Absolute Accuracy RSRQ | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151834 | 1119 | 1 | Test Tolerances for 9.2.4A.2 Relative Accuracy RSRQ | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151835 | 1121 | 1 | Uncertainties to Annex F for above 3GHz RRM test cases | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151838 | 1127 | 1 | Update Test Tolerances and Test requirements for TS 36.521-3 Test cases 4.2.3, 4.2.4, 4.2.5, 4.2.6 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151844 | 1128 | 1 | Uncertainties and Test Tolerances for Rel-12 Improved RSRP accuracy | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151846 | 1159 | 1 | Update Uncerts and Test Tools for TCs 8.11.1+8.11.2 for frequencies above 3GHz 36.521-3 CR | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151848 | 1160 | 1 | Update Uncerts and Test Tools for TC 8.11.3 for frequencies above 3GHz 36.521-3 CR | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151850 | 1161 | 1 | Update Uncerts and Test Tools for TC 8.11.4 for frequencies above 3GHz 36.521-3 CR | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151861 | 1171 | 1 | Addition of new RRM TC 8.16.25 E-UTRAN TDD-FDD CA Event Triggered Reporting on Deactivated SCell with PCell interruption in Non-DRX with PCell in FDD | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151862 | 1172 | 1 | Addition of new RRM TC 8.16.26 E-UTRAN TDD-FDD CA Event Triggered Reporting on Deactivated SCell with PCell interruption in Non-DRX with PCell in TDD | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151864 | 1123 | 1 | Corrections to RRM test cases for Core Alignment | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151866 | 1132 | 1 | New TC: E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151867 | 1133 | 1 | New TC: E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions for 20 MHz + 20 MHz bandwidth | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151868 | 1134 | 1 | New TC: E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions for 20 MHz + 10 MHz bandwidth | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151869 | 1135 | 1 | Cell configuration mapping for Rel 12 and forward RSRP relative accuracy RRM cases | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151870 | 1136 | 1 | Cell configuration mapping for Rel 12 and forward FDD absolute RSRP accuracy RRM cases | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151871 | 1137 | 1 | New TCs: R12 RSRP accuracy in eICIC | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151872 | 1138 | 1 | New TCs: R12 RSRP accuracy in FeICIC | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151873 | 1139 | 1 | New TCs: RSRP absolute accuracy for FDD E-UTRA CA | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151874 | 1140 | 1 | New TCs: R12 inter frequency relative accuracy of RSRP | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151875 | 1141 | 1 | Update of R12 CA RSRP measurement accuracy test cases | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151876 | 1142 | 1 | Update of R12 RSRP inter frequency measurement accuracy test cases | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151877 | 1143 | 1 | Update of R12 RSRP intra frequency measurement accuracy test cases | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151878 | 1144 | 1 | Update test applicabilities for R8 inter frequency relative accuracy of RSRP | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151879 | 1145 | 1 | Update test applicabilities for R9 inter frequency relative accuracy of RSRP | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151880 | 1146 | 1 | Update test applicabilities of R10 RSRP measurement accuracy | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151882 | 1147 | 1 | Update test applicabilities of R11 RSRP measurement accuracy | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151883 | 1148 | 1 | Update FDD CA RSRP Accuracy requirements | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151904 | 1174 | 1 | Correction to the titles of 2UL CA RRM test cases | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151905 | 1149 | 1 | Correction of allowed CQI report delays in sCell Activation Tests | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151906 | 1154 | 1 | Correction to feICIC Radio link monitoring test cases | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151915 | 1153 | 1 | Correction to test case 9.2.10.1 of 36.521-3 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151916 | 1175 | 1 | Correction to FeICIC measurement performance test cases and Annex H.3 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151917 | 1177 | - | Update Test Tolerances and Test requirements for TS 36.521-3 Test case 9.1.5.1 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151918 | 1178 | - | Update Test Tolerances and Test requirements for TS 36.521-3 Test case 9.1.5.2 | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151923 | 1169 | 1 | Addition of new RRM TC 8.16.23 E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151924 | 1170 | 1 | Addition of new RRM TC 8.16.24 E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151925 | 1173 | 1 | Applicability for newly added TDD-FDD CA RRM test cases | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-151939 | 1152 | 1 | Addition of FDD-TDD RSRP accuracy test cases for FDD-TDD CA. | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-152010 | 1151 | 1 | Correction to RLM test in feICIC | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-152015 | 1126 | 2 | Corrections to Derivation of Test Requirements for above 3GHz RRM test cases | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-152130 | 1110 | 1 | CA RRM: Corrections to cell configuration mapping table | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-152131 | 1156 | 1 | Correction to Event Triggered Reporting test in feICIC | 12.5.0 | 12.6.0 |
| 2015-06 | RAN#68 | R5-152134 | 1157 | 2 | Clarification of 2DL CA RRM inter-frequency tests with smaller channels bandwidth | 12.5.0 | 12.6.0 |
| 2015-09 | RAN#69 | R5-153094 | 1183 | - | Addition of cell configuration mapping for RRM test cases 8.16.17 and 8.16.18 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153095 | 1184 | - | Correction of cell configuration mapping for RRM cases | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153096 | 1185 | - | Correction of RMC used in UE Timing Advance Adjustment Accuracy Test for SCell in sTAG | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153164 | 1193 | - | New TC: E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20MHz + 10MHz bandwidth | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153167 | 1194 | - | Cell configuration mapping for newly introduced 20+20MHz and 20+10MHz cases | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153176 | 1197 | - | Addition of missing titles to clause 8 RRM test cases | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153177 | 1198 | - | Update Test Tolerances and Test requirements for Test case 4.3.1.1 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153179 | 1199 | - | Update Test Tolerances and Test requirements for Test case 4.3.4.1 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153181 | 1200 | - | Update Test Tolerances and Test requirements for Test cases 5.1.3+5.1.4+5.1.7+5.1.8 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153249 | 1204 | - | CA RRM: Clarification of PHICH configuration | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153250 | 1205 | - | RRM: Minor correction to TC 8.7.2 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153251 | 1206 | - | Uncertainties and Test tolerances for TS 36.521-3 Test case 8.20.2B | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153310 | 1209 | - | Correction of L2G PSHO applicability for TS 36.521-3 spec | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153357 | 1210 | - | Correction to RLM tests for eICIC | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153358 | 1211 | - | Correction to initial conditions in event triggered reporting tests | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153359 | 1212 | - | Correction to RSRP and RSRQ test cases | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153387 | 1215 | - | Correction to applicability of feICIC RRM test cases. | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153461 | 1222 | - | New TC: 8.20.2A for RRM | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153471 | 1226 | - | Update of test applicabilites for Rel-10 CA RSRP relative accuracy tests | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153473 | 1227 | - | Update of test applicabilites for Rel-11 CA RSRP relative accuracy tests | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153475 | 1228 | - | Update of Annex E for RRM 9.1.6.2_1 and 9.1.7.2_1 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153532 | 1233 | - | Update Uncertainties for Test case 5.2.7 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153544 | 1234 | - | Update Uncertainties for Test cases 5.2.8 and 5.2.9 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153547 | 1235 | - | Update Uncertainties for Test case 5.2.10 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153553 | 1236 | - | Correction to 8.16.17 and 8.16.18 activation and deactivation of known Scell | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153687 | 1239 | - | Uncerts and Test Tools for TCs 7.1.3+7.1.4 36.521-3 CR | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153688 | 1240 | - | Cell mapping for new RRM TCs 7.1.3_1+7.1.4_1 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153812 | 1190 | 1 | New TC: E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz +20MHz bandwidth | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153814 | 1224 | 1 | New TCs: FDD Rel-12 CA RSRP relative accuracy tests | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153815 | 1225 | 1 | New TCs: TDD Rel-12 CA RSRP relative accuracy tests | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153816 | 1241 | 1 | Addition of new RRM TCs 7.1.3_1+7.1.4_1 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153818 | 1186 | 1 | Correction of descriptions for test cases 8.16.18 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153819 | 1187 | 1 | Update editor’s note for RRM test case 8.16.17 and 8.16.18 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153829 | 1201 | 1 | Introduction of 3DL CA Event triggered reporting Test cases 8.16.27 and 8.16.28 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153830 | 1202 | 1 | Introduction of 3DL CA RSRP accuracy Test cases 9.1.37 and 9.1.38 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153831 | 1203 | 1 | Introduction of 3DL Pcell in FDD RSRQ for E-UTRAN in Carrier Aggregation | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153832 | 1207 | 1 | Addition of new TC of 3DL FDD RSRP for E-UTRAN in Carrier Aggregation | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153833 | 1208 | 1 | Addition of new TC of 3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153834 | 1217 | 1 | Updates to RRM TCs 8.16.23, 8.16.24, 8.16.25 and 8.16.26 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153871 | 1229 | 1 | New TCs: 9.1.40 and 9.1.40_1 TDD RSRP Accuracy for 3DL CA | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153872 | 1230 | 1 | New TCs: 9.2.39 TDD RSRQ Accuracy for 3DL CA | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153898 | 1218 | 1 | Addition of new TC 8.16.31 E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153899 | 1219 | 1 | Addition of new TC 8.16.32 E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153900 | 1220 | 1 | Addition of new TC 8.16.33 E-UTRAN FDD 3DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153901 | 1221 | 1 | Addition of new TC 8.16.34 E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153930 | 1188 | 1 | New TC: E-UTRAN TDD - UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz bandwidth | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153931 | 1189 | 1 | New TC: E-UTRAN TDD - UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz bandwidth | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153934 | 1195 | 1 | Correction to applicability of TD-LTE to UTRA TDD connected mode measurements | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153936 | 1191 | 1 | New TC: E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz + 20MHz bandwidth | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153937 | 1192 | 1 | New TC: E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz + 10MHz bandwidth | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153938 | 1196 | 1 | Correction to cell mapping of eICIC Chapter 9 test cases | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153939 | 1213 | 1 | Clarification of neighbour cell frequency for intra-band non-contiguous CA | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153940 | 1223 | 1 | Update of minimum requirements for relative RSRP Accuracy TC 9.1.6.2 and 9.1.7.2 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-153943 | 1238 | 1 | Introduction of new 3DL CA Test cases 8.16.30 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-154000 | 1179 | 1 | Introduction of TC 8.16.38 3DL TDD CA activation and deactivation of known SCell in non-DRX | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-154001 | 1182 | 1 | Introduction of 3 DL TDD Absolute/Relative RSRQ for E-UTRAN in Carrier Aggregation | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-154028 | 1214 | 1 | Update of Annex E for 3DL CA RRM | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-154029 | 1237 | 2 | Introduction of new 3DL CA Test cases 8.16.29 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-154033 | 1242 | 1 | Correction to relative measurement test cases of feICIC RRM test cases | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-154049 | 1243 | - | CA RRM: Additions to cell mapping Table E-1 | 12.6.0 | 12.7.0 |
| 2015-09 | RAN#69 | R5-154058 | 1244 | - | eICIC RRM: Corrections to cell mapping Table E-1 | 12.6.0 | 12.7.0 |
| 2015-12 | RAN#70 | R5-155108 | 1258 | - | Correction to test procedure of inter-frequency autonomous gaps test cases | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155134 | 1262 | - | Updating Annex E and F with newly introduced test cases for MTC | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155135 | 1263 | - | Adding RMCs for MTC to Annex A | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155136 | 1264 | - | Adding abbreviations and SS HD-FDD testing information | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155139 | 1266 | - | Uncertainties and Test tolerances for Test cases 4.3.2 and 4.3.4.2 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155141 | 1267 | - | Uncertainties and Test tolerances for Test cases 4.4.1 and 4.4.2 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155143 | 1268 | - | Uncertainties and Test tolerances for Test cases 4.5.1.1 and 4.5.2.1 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155145 | 1269 | - | Uncertainties and Test tolerances for Test case 8.20.2A | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155147 | 1270 | - | Uncertainties and Test tolerances for Test case 8.20.4A | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155149 | 1271 | - | Uncertainties and Test tolerances for Test case 8.20.4B | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155197 | 1286 | - | Uncertainties and Test Tolerance for E-UTRAN TDD - UE Timing Advanced Adjustment Accuracy Test for SCell in sTAG | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155270 | 1297 | - | Correction to test case 7.1.2 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155399 | 1304 | - | Addition of cell configuration mapping for newly introduced MTC tests | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155427 | 1305 | - | Uncertainties and Test Tolerances for 3DL FDD RSRP for E-UTRAN in Carrier Aggregation | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155429 | 1307 | - | Update to 3DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155437 | 1310 | - | Removable of wait time after activation of Scell | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155574 | 1319 | - | Incorrect Io values for eICIC tests | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155680 | 1328 | - | Adding new RRM test case 8.16.35 for 3DL CA | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155681 | 1329 | - | Adding new RRM test case 8.16.36 for 3DL CA | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155770 | 1330 | - | Introduction of test case 9.1.41.1 - FD-FDD Intra Frequency Absolute RSRP Accuracy for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155771 | 1331 | - | Introduction of test case 9.1.41.2 - FD-FDD Intra Frequency Relative RSRP Accuracy for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155772 | 1332 | - | Introduction of test case 9.1.42.1 - HD-FDD Intra Frequency Absolute RSRP Accuracy for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155773 | 1333 | - | Introduction of test case 9.1.42.2 - HD-FDD Intra Frequency Relative RSRP Accuracy for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155774 | 1334 | - | Introduction of test case 9.1.43.1 - TDD Intra Frequency Absolute RSRP Accuracy for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155775 | 1335 | - | Introduction of test case 9.1.43.2 - TDD Intra Frequency Relative RSRP Accuracy for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155776 | 1336 | - | Introduction of cell configuration mapping for Intra Frequency RSRP Accuracy for UE category 0 Test Cases | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155805 | 1256 | 1 | Update minimum requirements for test case 8.1.1 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155806 | 1265 | 1 | Updating and aligning of RMC tables in 3GPP TS36.521-3 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155820 | 1250 | 1 | Introduction of new test case 6.1.8 MTC | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155821 | 1252 | 1 | Introduction of new test case 8.1.12 MTC | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155822 | 1253 | 1 | Introduction of new test case 8.1.13 MTC | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155823 | 1254 | 1 | Introduction of new test case 8.1.17 MTC | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155824 | 1255 | 1 | Introduction of new test case 8.1.18 MTC | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155825 | 1259 | 1 | Adding new MTC handover test case 5.1.10 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155826 | 1260 | 1 | Adding new MTC handover test case 5.1.11 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155827 | 1261 | 1 | Adding new MTC handover test case 5.1.12 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155828 | 1272 | 1 | Adding new MTC FD-FDD RLM test case for out-of-sync and definition update | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155829 | 1273 | 1 | Adding new MTC FD-FDD RLM test case for in-sync | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155830 | 1274 | 1 | Adding new MTC FD-FDD RLM test case for out-of-sync in DRX | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155831 | 1275 | 1 | Adding new MTC FD-FDD RLM test case for in-sync in DRX | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155832 | 1276 | 1 | Adding new MTC HD-FDD RLM test case for out-of-sync | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155833 | 1277 | 1 | Adding new MTC HD-FDD RLM test case for in-sync | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155834 | 1278 | 1 | Adding new MTC HD-FDD RLM test case for out-of-sync in DRX | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155835 | 1279 | 1 | Adding new MTC HD-FDD RLM test case for in-sync in DRX | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155836 | 1280 | 1 | Adding new MTC TDD RLM test case for out-of-sync | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155837 | 1281 | 1 | Adding new MTC TDD RLM test case for in-sync | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155838 | 1282 | 1 | Adding new MTC TDD RLM test case for out-of-sync in DRX | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155839 | 1283 | 1 | Adding new MTC TDD RLM test case for in-sync in DRX | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155862 | 1284 | 1 | Update to RSRP accuracy test cases for FDD-TDD CA | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155863 | 1299 | 1 | Update of message contents for 3DL CA RSRP and RSRQ Accuracy tests | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155864 | 1309 | 1 | Correction to test applicability in 3DL CA RRM test cases | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155866 | 1315 | 1 | Clarification on frequency swapping and references to connection diagrams in 3DL CA RRM test cases | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155867 | 1316 | 1 | Addition of missing applicability release to test case 9.2.39 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155868 | 1320 | 1 | Introduction of new 3DL CA RRM test case TC 8.16.41 "3 DL FDD CA activation and deactivation of unknown SCell in non-DRX" | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155869 | 1327 | 1 | Updating 3DL CA test cases 8.16.29 and 8.16.30 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155878 | 1288 | 1 | Uncertainties and Test Tolerances for 3DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX Test cases | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155879 | 1290 | 1 | Uncertainties and Test Tolerances for 3DL E-UTRA for Carrier Aggregation RSRQ Accuracy Test cases | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155880 | 1321 | 1 | Uncerts and Test Tools for TCs 8.16.31+8.16.32+8.16.33+8.16.34 36.521-3 CR | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155881 | 1257 | 1 | Updated to TC 7.1.4A | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155888 | 1285 | 1 | Update RSRQ and reported values for serving cell Intra Frequency Absolute RSRQ Accuracy Test cases | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155889 | 1287 | 1 | Uncertainties and Test Tolerances for 2DL CA activation and deactivation of known SCell in non-DRX Test cases | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155896 | 1293 | 1 | Add new LC MTC test case 8.1.19 E-UTRAN FD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155897 | 1294 | 1 | Add new LC MTC test case 8.1.20 E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0. | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155898 | 1295 | 1 | Add new LC MTC test case 8.1.21 E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-155899 | 1296 | 1 | Add new LC MTC test case 8.1.22 E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156009 | 1298 | 1 | Update Test applicability for test case 7.1.7 and 7.2.5 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156015 | 1326 | 1 | Addition of UE transmit timing accuracy test case for PRoSe Direct discovery. | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156032 | 1289 | 1 | Uncertainties and Test Tolerances for 3DL E-UTRA for Carrier Aggregation RSRP Accuracy Test cases | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156033 | 1308 | 1 | Update of Annex E for 3DL CA RRM | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156034 | 1311 | 1 | Correction to 3DL CA RSRP/RSRQ test cases | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156051 | 1251 | 1 | Introduction of new test case 8.1.11 MTC | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156053 | 1291 | 1 | Add new LC MTC test case 6.1.6 E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156054 | 1292 | 1 | Add new LC MTC test case 6.1.7 E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0category 0 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156067 | 1322 | 1 | Addition of new SCE-L1 RRM TC 8.22.1 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156068 | 1323 | 1 | Addition of new SCE-L1 RRM TC 8.22.2 E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156069 | 1324 | 1 | Addition of new SCE-L1 RRM TC 8.22.3 E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156070 | 1325 | 1 | Addition of new SCE-L1 RRM TC 8.22.4 E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156128 | 1313 | 1 | Correction to message contents for RRM 3DL CA TC 9.2.41 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156129 | 1314 | 1 | Corrections to message contents for RRM 3DL CA TCs 8.16.27+8.16.28 | 12.7.0 | 12.8.0 |
| 2015-12 | RAN#70 | R5-156137 | 1301 | 2 | Update of test applicabilities for R12 RRM cases in 36.521-3 | 12.7.0 | 12.8.0 |
| 2016-03 | RAN#71 | R5-160121 | 1346 | - | Updates and Test Tolerances for 2DL FDD-TDD E-UTRA for Carrier Aggregation RSRP Accuracy Test cases | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160191 | 1349 | - | Updates and Test Tolerances for 2DL E-UTRA for Carrier Aggregation RSRQ Accuracy Test cases | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160200 | 1350 | - | Band 66 update of 3DL E-UTRA for CA RSRP Accuracy Test cases | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160204 | 1351 | - | Correction to test case 8.20.2B | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160207 | 1353 | - | Correction to test case 8.16.17 and 8.16.18 | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160221 | 1356 | - | Uncertainties and TT for 8.16.23 and 8.16.24 TDD-FDD CA event triggered reporting under deactivated SCell in non-DRX with PCell in FDD/TDD 36.521-3 | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160222 | 1357 | - | Uncertainties and TT for 8.16.25 and 8.16.26 TDD-FDD CA event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD/TDD 36.521-3 | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160237 | 1358 | - | Condition definitions for discovery measurements and measurement accuracy requirements | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160257 | 1363 | - | Addition of new SCE-L1 RRM TC 8.22.9 E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160258 | 1364 | - | Addition of new SCE-L1 RRM TC 8.22.10 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160261 | 1367 | - | Addition of new SCE-L1 RRM TC 9.1.25 FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160262 | 1368 | - | Addition of new SCE-L1 RRM TC 9.1.26 TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160263 | 1369 | - | Addition of new SCE-L1 RRM TC 9.1.27 FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160264 | 1370 | - | Addition of new SCE-L1 RRM TC 9.1.28 TDD—TDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160265 | 1371 | - | Addition of new SCE-L1 RRM TC 9.1.33 FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160266 | 1372 | - | Addition of new SCE-L1 RRM TC 9.1.34 TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160267 | 1373 | - | Addition of new SCE-L1 RRM TC 9.2.28 FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160268 | 1374 | - | Addition of new SCE-L1 RRM TC 9.2.29 TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160269 | 1375 | - | Addition of new SCE-L1 RRM TC 9.2.30 FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160270 | 1376 | - | Addition of new SCE-L1 RRM TC 9.2.31 TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160271 | 1377 | - | Addition of new SCE-L1 RRM TC 9.2.32 FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160272 | 1378 | - | Addition of new SCE-L1 RRM TC 9.2.33 TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160443 | 1382 | - | Introduction of new Dual Connectivity test case 8.23.1 | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160444 | 1383 | - | Introduction of new Dual Connectivity test case 8.23.2 | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160445 | 1384 | - | Introduction of new Dual Connectivity test case 8.23.3 | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160446 | 1385 | - | Introduction of new Dual Connectivity test case 8.23.4 | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160447 | 1386 | - | Introduction of new Dual Connectivity test case 8.23.5 | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160448 | 1387 | - | Introduction of new Dual Connectivity test case 8.23.6 | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160450 | 1389 | - | Update to MTC test cases 8.1.11, 8.1.12, 8.1.13, 8.1.17, 8.1.18 | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160452 | 1391 | - | Update to test cases 8.16.17 | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160497 | 1397 | - | Correction to event triggered tests without measurement gaps | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160498 | 1398 | - | Correction to RLM tests in feICIC | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160556 | 1405 | - | CA RRM: Inconsistent message contents in 3CC event triggered tests | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160558 | 1407 | - | CA RRM: Missing message contents in 3CC TCs 9.1.37 and 9.1.38 | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160559 | 1408 | - | RRM: Correction of applicability of TCs 9.3.2 and 9.4.2 | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160572 | 1409 | - | Updating TS36.521-3 clause 3.5.1 for band 66 | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160574 | 1410 | - | Updating Annex I with band group FDD_B | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160610 | 1416 | - | Antenna configuration principle for 36.521-3 | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160702 | 1417 | - | Introducing DC RRM test case 7.3.38 | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160703 | 1418 | - | Introducing DC RRM test case 7.3.39 | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160704 | 1419 | - | Introducing DC RRM test case 7.3.40 | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160705 | 1420 | - | Updating References, Definitions and Abbreviations for DC in 36.521-3 | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160821 | 1361 | 1 | Addition of new SCE-L1 RRM TC 8.22.6 E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160822 | 1362 | 1 | Addition of new SCE-L1 RRM TC 8.22.8 E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160823 | 1365 | 1 | Addition of new SCE-L1 RRM TC 8.22.11 E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160824 | 1366 | 1 | Addition of new SCE-L1 RRM TC 8.22.12 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160833 | 1406 | 1 | CA RRM: Ambiguous description of relative RSRQ measurements in 3CC tests | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160834 | 1395 | 1 | Correction to test frequency configuration for 3DL CA RRM | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160847 | 1352 | 1 | Correction to test case 7.1.4 and 7.1.4_1 | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160872 | 1402 | 1 | Updating Annex F for E-UTRAN 3DL CA Activation and Deactivation of Known SCell in Non-DRX test cases in 3GPP TS36.521-3 | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160876 | 1388 | 1 | Uncertainties and Test Tolerances for E-UTRAN UE Transmit Timing Accuracy Tests for SCell in sTAG in FDD/TDD Test cases | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160883 | 1390 | 1 | Update to RRM test cases 7.1.3, 7.1.3_1, 7.1.4, 7.1.4_1, 7.1.4A for TT and Uncerts | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160884 | 1414 | 1 | Update of HO testcases for MTC | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160886 | 1339 | 2 | Introduction of TC 8.1.15 E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0MTC | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160887 | 1338 | 2 | Add new LC MTC test case 8.1.14 E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0 | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160888 | 1340 | 2 | Introduction of TC 8.1.16 E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0 | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160889 | 1341 | 2 | Introduction of TC 8.2.7 E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0 | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160890 | 1342 | 2 | Introduction of TC 8.2.8 E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0 | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160891 | 1343 | 2 | Introduction of TC 9.2.42.1 FD-FDD Intra Frequency Absolute RSRQ Accuracy for UE category 0 | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160892 | 1344 | 2 | Introduction of TC 9.2.43.1 HD-FDD Intra Frequency Absolute RSRQ Accuracy for UE category 0 | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160893 | 1345 | 2 | Introduction of TC 9.2.44.1 TDD Intra Frequency Absolute RSRQ Accuracy for UE category 0 | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-160894 | 1415 | 1 | Update of RLM testcases for MTC | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-161002 | 1394 | 1 | Correction to RRC message exceptions for 3DL CA RSRP/RSRQ | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-161007 | 1403 | 1 | Addition of test cases for transmission timing accuracy and Interruptions for D2D | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-161037 | 1359 | 1 | Addition of new RRM TC 8.22.5 E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-161038 | 1360 | 1 | Addition of new RRM TC 8.22.7 E-UTRAN FDD-FDD inter-frequency event triggered reporting for DRS measurement based on CSI-RS with DRX | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-161051 | 1411 | 1 | Correcting the applicability of test cases 9.5.1 and 9.5.2 | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-161068 | 1392 | 1 | Update to test cases 8.16.17A and 8.16.18A | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-161094 | 1393 | 2 | Correction to Event Triggered Reporting test under Deactivated SCells | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-161095 | 1396 | 2 | Correction to 3DL CA with deactivated SCell with PCell and SCell interruptions | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-161096 | 1400 | 1 | Updating RRM test case 8.16.35 | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-161097 | 1401 | 2 | Updating RRM test case 8.16.36 | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-161098 | 1354 | 2 | New TC 8.16.39 E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-161099 | 1355 | 2 | New TC 8.16.40 E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-161100 | 1380 | 2 | Addition of new test case for 3DL FDD CA activation and deactivation of known cell | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-161106 | 1347 | 2 | Test Tolerances for TC 8.16.38 3DL TDD CA activation and deactivation of known SCell in non-DRX | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-161117 | 1379 | 2 | Updates of FDD 2DL CA activation and deactivation test case | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-161118 | 1399 | 2 | Correction to 9.1.15.1 and 9.1.15.2 RSRP tests for feICIC | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-161122 | 1413 | 3 | Correction of test procedure for 3DL CA RRM test case TC 8.16.41 "3 DL FDD CA activation and deactivation of unknown SCell in non-DRX" | 12.8.0 | 12.9.0 |
| 2016-03 | RAN#71 | R5-161123 | 1412 | 3 | Introduction of new 3DL CA RRM test case TC 8.16.42 "3 DL TDD CA activation and deactivation of unknown SCell in non-DRX" | 12.8.0 | 12.9.0 |
| 2016-06 | RAN#72 | R5-162186 | 1422 | - | Editorial correction uplink absolute power measurement in Annex F.1.2 | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162190 | 1423 | - | Update of test tolerance for UE ProSe discovery transmission timing | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162193 | 1424 | - | Update of Test Tolerance analyses for 7.5.1 | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162278 | 1425 | - | Correction to Table number for using MAC-MainConfig-RBC message | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162286 | 1430 | - | Update of RRM RLM test cases for Cat 0 | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162288 | 1431 | - | Update of Annex F for Cat 0 RLM test cases | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162310 | 1432 | - | RRM: Correction to TCs 7.1.7A and 7.1.7B | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162311 | 1433 | - | RRM: Corrections to ri-ConfigIndex in 3CC test cases | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162312 | 1434 | - | RRM: Missing release applicability of inter-mode 3CC test cases | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162313 | 1435 | - | RRM: Clarification of message content in TC 8.16.36 | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162321 | 1437 | - | Addition of test cases for SLSS initiation and cease | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162330 | 1439 | - | Introduction of TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162334 | 1442 | - | Correction of 3DL CA RRM test case 8.16.36 | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162336 | 1443 | - | Introduction of TDD-TDD inter frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162397 | 1447 | - | Uncertainties and Test tolerances for TS 36.521-3 Test cases 9.1.6.1_1 and 9.1.12.1_1 | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162399 | 1448 | - | Uncertainties and Test tolerances for TS 36.521-3 Test cases 9.1.6.2_1 and 9.1.12.2_1 | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162430 | 1450 | - | Update of RRM test cases 9.1.1.1, 9.1.2.1, 9.1.16.1 to include band 65, 66 | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162433 | 1451 | - | Update of RRM test cases 9.1.1.1_1, 9.1.2.1_1, 9.1.16.1_1 to include band 65, 66 | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162436 | 1452 | - | Update of RRM test cases 9.1.1.2, 9.1.2.2, 9.1.16.2 to include band 65, 66 | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162439 | 1453 | - | Update of RRM test cases 9.1.3.1, 9.1.4.1, 9.1.17.1 to include band 65, 66 | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162600 | 1461 | - | Correction to test applicability in 3DL CA activation and deactivation TCs | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162830 | 1463 | 1 | Correction to 3DL CA activation and deactivation TCs | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162831 | 1454 | 1 | Correction to applicability for EUTRA TDD to UTRA TDD Son test case | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162833 | 1438 | 1 | Introduction of FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162834 | 1441 | 1 | Introduction of FDD-FDD inter frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162835 | 1444 | 1 | Introduction of FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162836 | 1445 | 1 | Introduction of TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162881 | 1436 | 1 | Addition of ProSe cell selection test cases | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162891 | 1421 | 1 | Uncertainties and Test Tolerances for MTC RRM TC 8.1.11, 8.1.12, 8.1.13, 8.1.17 and 8.1.18 | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162894 | 1429 | 1 | Test Tolerance updates for MTC test cases in clause 6 | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162895 | 1457 | 1 | Add Test requirements for TS 36.521-3 Test case 8.22.1 and 8.22.2 | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162897 | 1458 | 1 | Add Test requirements for TS 36.521-3 Test case 8.22.3 and 8.22.4 | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162957 | 1426 | 1 | Update to Dual Connectivity RRM test case 8.23.4 | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162958 | 1427 | 1 | Update to Dual Connectivity RRM test case 8.23.5 | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162959 | 1428 | 1 | Update to Dual Connectivity RRM test case 8.23.6 | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162960 | 1446 | 1 | Added new clause for Synchronous and Asynchronous DC test principles | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162961 | 1456 | 1 | Update to Dual Connectivity RRM test case 8.23.2 | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162962 | 1467 | 1 | Update to Dual Connectivity RRM test case 8.23.3 | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162963 | 1468 | 1 | Update of DC RLM test cases | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162964 | 1469 | 1 | Adding new RLM test case 7.3.41 | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162965 | 1470 | 1 | Adding new RLM test case 7.3.42 | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-162966 | 1471 | 1 | Adding new RLM test case 7.3.43 | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-163111 | 1455 | 1 | Update to Dual Connectivity RRM test case 8.23.1 | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-163128 | 1459 | 1 | Add Test requirements for TS 36.521-3 Test case 9.1.25 and 9.1.26 | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-163197 | 1462 | 2 | Correction to duration of T3 in 3 DL CA Event Triggered Reporting on Deactivated Scell | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-163134 | 1464 | 1 | Clean up on RRM test cases in chapter 7, 8, 9 | 12.9.0 | 12.10.0 |
| 2016-06 | RAN#72 | R5-163135 | 1465 | 1 | Correction to event triggered reporting under deactivated SCell(10MHz+5MHz and 20MHz+10MHz) | 12.9.0 | 12.10.0 |
| 2016-09 | RAN#73 | R5-165130 | 1472 | - | Parameter correction for TC 9.2.16.1 | 12.10.0 | 12.11.0 |
| 2016-09 | RAN#73 | R5-165166 | 1477 | - | Update Test Tolerances and Test requirements for TS 36.521-3 Test cases 9.1.3.1_1 and 9.1.4.1_1 | 12.10.0 | 12.11.0 |
| 2016-09 | RAN#73 | R5-165168 | 1478 | - | Update Test Tolerances and Test requirements for TS 36.521-3 Test cases 9.1.3.2_1 and 9.1.4.2_1 | 12.10.0 | 12.11.0 |
| 2016-09 | RAN#73 | R5-165296 | 1482 | - | Band 66 update of Inter-freq relative RSRP accuracy Test cases | 12.10.0 | 12.11.0 |
| 2016-09 | RAN#73 | R5-165389 | 1490 | - | Update of test tolerance for UE ProSe direct communication transmission timing | 12.10.0 | 12.11.0 |
| 2016-09 | RAN#73 | R5-165428 | 1494 | - | Modification of measurement report configuration for TC 8.1.8 and 8.2.6 | 12.10.0 | 12.11.0 |
| 2016-09 | RAN#73 | R5-165527 | 1496 | - | Add Test requirements for TS 36.521-3 Test case 8.22.9 and 8.22.10 | 12.10.0 | 12.11.0 |
| 2016-09 | RAN#73 | R5-165528 | 1497 | - | Add Test requirements for TS 36.521-3 Test case 9.2.28 and 9.2.29 | 12.10.0 | 12.11.0 |
| 2016-09 | RAN#73 | R5-165529 | 1498 | - | Add Test requirements for TS 36.521-3 Test case 9.2.30 and 9.2.31 | 12.10.0 | 12.11.0 |
| 2016-09 | RAN#73 | R5-165530 | 1499 | - | Add Test requirements for TS 36.521-3 Test case 9.2.32 and 9.2.33 | 12.10.0 | 12.11.0 |
| 2016-09 | RAN#73 | R5-165577 | 1507 | - | Update Annex F for TS 36.521-3 Test case 9.1.27, 9.1.28, 9.1.33 and 9.1.34 | 12.10.0 | 12.11.0 |
| 2016-09 | RAN#73 | R5-165643 | 1511 | - | Correction to applicability for TDD-UTRAN TDD SON | 12.10.0 | 12.11.0 |
| 2016-09 | RAN#73 | R5-165664 | 1513 | - | Correction to test parameters of 3DL CA event triggered report test cases for 8.16.31, 8.16.32, 8.16.33 and 8.16.34 | 12.10.0 | 12.11.0 |
| 2016-09 | RAN#73 | R5-165665 | 1514 | - | Clarification of test parameters for RRM asymmetric bandwidth 2CA test cases | 12.10.0 | 12.11.0 |
| 2016-09 | RAN#73 | R5-165666 | 1515 | - | Clarification of test applicability for 2DL CA with intra-band non-contiguous | 12.10.0 | 12.11.0 |
| 2016-09 | RAN#73 | R5-165667 | 1516 | - | Clean up on CA RRM test cases 8.16.25 and 9.1.40_1 | 12.10.0 | 12.11.0 |
| 2016-09 | RAN#73 | R5-165668 | 1517 | - | Update of cell configuration mappings for FDD/TDD 3CA band combination | 12.10.0 | 12.11.0 |
| 2016-09 | RAN#73 | R5-165669 | 1518 | - | Correction to the test applicability for test cases 8.20.4, 8.20.4A and 8.20.4B | 12.10.0 | 12.11.0 |
| 2016-09 | RAN#73 | R5-165670 | 1519 | - | Clean up on RRM test cases 8.7.2 and 8.11.4 | 12.10.0 | 12.11.0 |
| 2016-09 | RAN#73 | R5-165851 | 1537 | - | Removing redundant Editor’s note from Clause 9 in 36.521-3 | 12.10.0 | 12.11.0 |
| 2016-09 | RAN#73 | R5-165852 | 1538 | - | Adding antenna connection diagram references to RRM DC RLM test cases | 12.10.0 | 12.11.0 |
| 2016-09 | RAN#73 | R5-165853 | 1539 | - | Updating Annex E with DC RLM test cases 7.3.38 - 7.3.43 | 12.10.0 | 12.11.0 |
| 2016-09 | RAN#73 | R5-165861 | 1540 | - | Update of definitions for IncMon | 12.10.0 | 12.11.0 |
| 2016-09 | RAN#73 | R5-166024 | 1500 | 1 | Correction of triggering event for SCE test case 8.22.5 and 8.22.6 | 12.10.0 | 12.11.0 |
| 2016-09 | RAN#73 | R5-166025 | 1501 | 1 | Introduce Cell configuration mapping for SCE RRM test cases | 12.10.0 | 12.11.0 |
| 2016-09 | RAN#73 | R5-166098 | 1484 | 1 | Update of test tolerance for UE ProSe direct communication SLSS initiation and cease | 12.10.0 | 12.11.0 |
| 2016-09 | RAN#73 | R5-166100 | 1512 | 1 | Correction to 3DL CA Activation and Deactivation test cases | 12.10.0 | 12.11.0 |
| 2016-09 | RAN#73 | R5-165575 | 1505 | - | Update Test requirement for TS 36.521-3 Test case 9.1.27and 9.1.28 | 12.10.0 | 12.11.0 |
| 2016-09 | RAN#73 | R5-165576 | 1506 | - | Update Test requirement for TS 36.521-3 Test case 9.1.33 and 9.1.34 | 12.10.0 | 12.11.0 |
| 2016-09 | RAN#73 | R5-166305 | 1536 | 1 | Adding CQI reporting periodicity to MTC test cases | 12.10.0 | 12.11.0 |
| 2016-09 | RAN3#73 | R5-166306 | 1492 | 1 | Uncertainties and Test Tolerances for intra frequency absolute and relative CSI-RSRP accuracies in CRI-RS based discovery signal Aggregation | 12.10.0 | 12.11.0 |
| 2016-09 | RAN#73 | R5-166308 | 1527 | 1 | Add note to cell mapping table in Annex E for band 66 high range testing | 12.10.0 | 12.11.0 |
| 2016-09 | RAN#73 | R5-165385 | 1488 | - | Introduction of FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | 12.11.0 | 13.0.0 |
| 2016-09 | RAN#73 | R5-165388 | 1489 | - | Introduction of HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | 12.11.0 | 13.0.0 |
| 2016-09 | RAN#73 | R5-165390 | 1491 | - | Introduction of TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | 12.11.0 | 13.0.0 |
| 2016-09 | RAN#73 | R5-165505 | 1495 | - | Introduction of Band 45 into 36.521-3 | 12.11.0 | 13.0.0 |
| 2016-09 | RAN#73 | R5=165570 | 1502 | - | Update Test requirement for TS 36.521-3 Test case 6.2.7 and 6.2.8 | 12.11.0 | 13.0.0 |
| 2016-09 | RAN#73 | R5-165571 | 1503 | - | Update Test requirement for TS 36.521-3 Test case 7.2.4 | 12.11.0 | 13.0.0 |
| 2016-09 | RAN#73 | R5-165572 | 1504 | - | Update Annex F for TS 36.521-3 Test case 6.2.7, 6.2.8 and 7.2.4 | 12.11.0 | 13.0.0 |
| 2016-09 | RAN#73 | R5-165739 | 1520 | - | Correction to Band 66 notes in E-UTRA band groups in 36.521-3 | 12.11.0 | 13.0.0 |
| 2016-09 | RAN#73 | R5-165836 | 1528 | - | Adding new eMTC test case 6.1.9 to 36.521-3 | 12.11.0 | 13.0.0 |
| 2016-09 | RAN#73 | R5-165839 | 1529 | - | Adding new eMTC test case 6.1.10 to 36.521-3 | 12.11.0 | 13.0.0 |
| 2016-09 | RAN#73 | R5-165840 | 1530 | - | Adding new eMTC test case 6.1.11 to 36.521-3 | 12.11.0 | 13.0.0 |
| 2016-09 | RAN#73 | R5-165841 | 1531 | - | Adding new eMTC test case 7.1.10 to 36.521-3 | 12.11.0 | 13.0.0 |
| 2016-09 | RAN#73 | R5-165842 | 1532 | - | Adding new eMTC test case 7.1.11 to 36.521-3 | 12.11.0 | 13.0.0 |
| 2016-09 | RAN#73 | R5-165844 | 1533 | - | Adding new eMTC RRM test cases to Annex E of 36.521-3 | 12.11.0 | 13.0.0 |
| 2016-09 | RAN#73 | R5-165845 | 1534 | - | Adding new eMTC RMCs to Annex A of 36.521-3 | 12.11.0 | 13.0.0 |
| 2016-09 | RAN#73 | R5-166089 | 1485 | 1 | Introduction of E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA | 12.11.0 | 13.0.0 |
| 2016-09 | RAN#73 | R5-166090 | 1487 | 1 | Introduction of E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | 12.11.0 | 13.0.0 |
| 2016-09 | RAN#73 | R5-166163 | 1474 | 1 | Introduction of eMTC RRM TC 4.2.12 | 12.11.0 | 13.0.0 |
| 2016-09 | RAN#73 | R5-166164 | 1475 | 1 | Introduction of eMTC RRM TC 4.2.13 | 12.11.0 | 13.0.0 |
| 2016-09 | RAN#73 | R5-166167 | 1486 | 1 | Introduction of E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA | 12.11.0 | 13.0.0 |
| 2016-09 | RAN#73 | R5-166182 | 1543 | 1 | Addition of RLM test cases for FD-FDD CE Mode A | 12.11.0 | 13.0.0 |
| 2016-09 | RAN#73 | R5-166175 | 1523 | 1 | New Test Case: 7.3.50: E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-Sync in DRX for UE Category M1 Configured in CEMode A | 12.11.0 | 13.0.0 |
| 2016-09 | RAN#73 | R5-166176 | 1524 | 1 | New test case: 7.3.51: E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 Configured in CEMode A | 12.11.0 | 13.0.0 |
| 2016-09 | RAN#73 | R5-166178 | 1526 | 1 | New test case: 7.3.55: E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A | 12.11.0 | 13.0.0 |
| 2016-09 | RAN#73 | R5-166183 | 1544 | 1 | Addition of RLM test cases for HD-FDD CE Mode A | 12.11.0 | 13.0.0 |
| 2016-09 | RAN#73 | R5-166316 | 1521 | 2 | New Test Case: 6.2.10: E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage | 12.11.0 | 13.0.0 |
| 2016-09 | RAN#73 | R5-166317 | 1522 | 2 | New Test Case: 6.2.11: E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage | 12.11.0 | 13.0.0 |
| 2016-09 | RAN#73 | R5-166318 | 1535 | 1 | Adding new OCNG pattern definitions for eMTC to Annex D of 36.521-3 | 12.11.0 | 13.0.0 |
| 2016-09 | RAN#73 | R5-166333 | 1525 | 2 | New Test Case: 7.3.54: E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE Category M1 Configured in CEMode A | 12.1.00 | 13.0.0 |
| 2016-12 | RAN#74 | R5-168061 | 1546 | F | New Reference Channel for TS 37.571-1 RSTD tests | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168131 | 1547 | F | Addition of new test case E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168132 | 1548 | F | Addition of new test case E-UTRAN TDD Radio Link Monitoring Test for In-sync for Cat-M1 UE in CEMode A | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168133 | 1549 | F | Addition of new test case E-UTRAN TDD Radio Link Monitoring Test for Out-of-Sync in DRX for UE Category M1 Configured in CEMode A | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168134 | 1550 | F | Addition of new test case E-UTRAN TDD Radio Link Monitoring Test for In-Sync in DRX for UE Category M1 Configured in CEMode A | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168156 | 1551 | F | Introduction of new definitions for NB-IoT in Chapter 3 | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168157 | 1552 | F | Introduction of Reference NPRACH Configurations for NB-IoT | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168158 | 1553 | F | Introduction of OCNG pattern for NB-IoT | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168159 | 1554 | F | Correction of NPDCCH RMC for NB-IoT | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168170 | 1558 | F | Uncertainties and Test Tolerances for inter frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168172 | 1559 | F | Uncertainties and Test Tolerances for FDD/TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168179 | 1565 | F | Uncertainties and Test Tolerances for intra frequency absolute and relative RSRP accuracies of Cat-M1 UE in CEModeA | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168185 | 1568 | F | Introduction of HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168197 | 1569 | F | Updating Annex F by adding clause 6 test cases for eMTC | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168199 | 1570 | F | Updating Annex F by adding clause 7 test cases for eMTC | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168200 | 1571 | F | Updating Annex E by adding test cases 6.2.10 and 6.2.11, eMTC | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168202 | 1572 | F | Adding Reference PRACH Configuration for eMTC into 36.521-3 | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168206 | 1576 | F | Updating eMTC RRM test case 7.1.10 | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168208 | 1577 | F | Updating eMTC RRM test case 7.1.11 | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168248 | 1585 | F | Addition of new eMTC RRM test cases to Annex E of 36.521-3 | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168251 | 1586 | F | Introduction of eMTC RRM new test case 4.2.14 | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168289 | 1588 | F | HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 In-Band mode under enhanced coverage | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168295 | 1590 | F | Adding new eMTC RRM test cases to Annex E | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168296 | 1591 | F | Introduction of new eMTC RRM test case 8.1.29 for category M1 | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168297 | 1592 | F | Introduction of new eMTC RRM test case 8.1.30 for category M1 | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168306 | 1594 | F | Updated to DC RRM inter-frequency event triggered reporting test cases | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168475 | 1600 | F | Uncertainties, Test Tolerances and RSRQ absolute accuracy requirements for MTC RRM TC 9.2.42.1, 9.2.43.1 and 9.2.44.1 | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168504 | 1604 | F | NB-IoT TA adjustment enhanced coverage | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168505 | 1605 | F | HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 under normal coverage | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168608 | 1619 | F | Update to CE Mode A RLM test cases | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168620 | 1620 | F | Updates to message contents for eMTC RRM TC 4.2.12 | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168621 | 1621 | F | Updates to message contents for eMTC RRM TC 4.2.13 | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168622 | 1622 | F | Test Tolerances for eMTC RRM TC 4.2.12 | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168623 | 1623 | F | Test Tolerances for eMTC RRM TC 4.2.13 | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168662 | 1626 | F | Introduction of Intra frequency handover Test case 5.1.13 for Cat-M1 UEs in CEModeA | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168663 | 1627 | F | Introduction of Intra frequency handover Test case 5.1.14 for Cat-M1 UEs in CEModeA | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168664 | 1628 | F | Introduction of Intra frequency handover Test case 5.1.15 for Cat-M1 UEs in CEModeA | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168678 | 1629 | F | Clarification of test parameters for RRM asymmetric bandwidth 2CA test cases | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168679 | 1630 | F | Correction to TDD CA Activation and Deactivation of known SCell in Non-DRX test cases | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168680 | 1631 | F | Correction to figure group for timing adjustment 2CA TCs | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168681 | 1632 | F | Correction to RRM TDD FDD 2CA Event Triggered Report test cases | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168682 | 1633 | F | Correction to Information Bit Payload for TDD PDSCH RMC | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168710 | 1636 | F | Adding new test case 7.3.44 DC RLM FDD-TDD | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168711 | 1637 | F | Adding new test case 7.3.45 DC RLM TDD-FDD | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168713 | 1638 | F | Adding new test case 7.3.46 DC RLM TDD-FDD | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168716 | 1639 | F | Adding new test case 7.3.47 DC RLM TDD-FDD | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168718 | 1640 | F | Adding new DC RLM TDD-FDD test cases into Annex E | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168732 | 1644 | F | Adding new test cases for 4 Rx RRM RLM FDD testing to annex E | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168810 | 1646 | F | Updating DC RLM RRM test cases | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168814 | 1648 | F | Updating DC RLM RRM test cases Annex F | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168830 | 1650 | F | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168836 | 1652 | F | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168848 | 1653 | F | Updating test cases 9.1.6, 9.1.7, 9.1.12, 9.1.13 To include band group FDD_B | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168854 | 1654 | F | Updating test cases 9.1.6.2, 9.1.7.2, 9.1.12.2, 9.1.13.2 To include band group FDD_B | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168880 | 1656 | F | Updating eMTC RRM test case 6.1.9 | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168881 | 1657 | F | Updating eMTC RRM test case 6.1.10 | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168882 | 1658 | F | Updating eMTC RRM test case 6.1.11 | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168938 | 1659 | F | Uncertainties and Test Tolerances for intra frequency absolute and relative RSRP accuracies for UE category 0 Aggregation | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168971 | 1661 | F | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169517 | 1634 | F | Correction to RRM TDD FDD 2CA measurement accuracy test cases | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169529 | 1556 | F | Introduction of Cell configuration mapping for NB-IoT | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169552 | 1610 | F | Addition of the new TC 9.1.44 4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169553 | 1611 | F | Addition of the new TC 9.2.45 4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169575 | 1578 | F | New Test Case: 6.2.13: E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169576 | 1579 | F | New Test Case: 6.2.14: E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169577 | 1580 | F | New Test Case: 6.2.15: E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169578 | 1581 | F | Update Annex F Test Tolerance for E-UTRAN FDD-FDD and HD-FDD Radio Link Monitoring Test in DRX for UE Category M1 Configured in CEMode A | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169579 | 1560 | F | Introduction of test cases 7.2.6 in 3GPP TS36.521-3 | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169580 | 1561 | F | Introduction of test cases 7.2.7 in 3GPP TS36.521-3 | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169581 | 1562 | F | Introduction of test cases 7.2.8 in 3GPP TS36.521-3 | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169582 | 1601 | F | New test case: 8.1.33 E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169583 | 1602 | F | New test case: 8.1.34 E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169584 | 1603 | F | New test case: 8.1.35 E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169585 | 1624 | F | Introduction of new eMTC RRM TC 4.2.15 | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169586 | 1635 | F | eMTC RRM TC 6.2.10 prach-StartingSubframe and numRepetitionPerPreambleAttempt should match | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169587 | 1647 | F | Introduction of E-UTRAN HD - FDD Intra-Frequency Case for Cat-M1 UE in Enhanced Coverage Test Case | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169588 | 1649 | F | Introduction of E-UTRAN TDD - TDD Intra-Frequency Case for Cat-M1 UE in Enhanced Coverage Test Case | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169594 | 1660 | F | Update to E-UTRAN DC intra-frequency event triggered reporting DC Test cases | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169595 | 1609 | F | Update of test tolerance for UE Initiation and Cease of SLSS for ProSe direct communication | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169599 | 1595 | F | Introduction of new eMTC RRM test case 9.1.57 for category M1 | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169600 | 1617 | F | Addition of a new test case 9.1.59 TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169601 | 1618 | F | Update Annex F for TS 36.521-3 Test case 9.1.57, 9.1.58 and 9.1.59 | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169609 | 1614 | F | Correction of triggering event for SCE test case 8.22.7 and 8.22.8 | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169641 | 1641 | F | Adding Antenna connection configuration guide for 4 Rx RRM testing | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169679 | 1642 | F | Adding new test case 7.3.1_1 for 4 Rx RRM RLM FDD testing | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169680 | 1643 | F | Adding new test case 7.3.2_1 for 4 Rx RRM RLM FDD testing | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169695 | 1555 | F | Correction of NPDSCH RMC for NB-IoT | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169698 | 1625 | F | Addition of in-sync RLM test cases for NB-IOT | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169711 | 1587 | F | Introduction of eMTC RRM new test case 6.2.12 | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169718 | 1615 | F | Correction of triggering event for SCE test case 8.22.11 and 8.22.12 | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169726 | 1606 | F | Uncertainties and Test Tolerances for MTC RRM TCs 8.1.19,8.1.20,8.1.21 and 8.2.22 | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-169727 | 1607 | F | Uncertainties and Test Tolerances for MTC RRM TCs 8.2.7 and 8.2.8 | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168404 | 1599 | F | Band 70 information to E-UTRA band group in 36.521-3 | 13.1.0 | 14.0.0 |
| 2017-01 | RAN#74 | - | - | - | corrections concerning the implementation of R5-168678, R5-168848, R5-169517 | 14.0.0 | 14.0.1 |
| 2017-03 | RAN#75 | R5-170588 | 1680 | - | Update test case for HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 In-Band mode under enhanced coverage | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-170589 | 1681 | - | Update test case for HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 under normal coverage | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-170590 | 1682 | - | Update test case for Timing Advance Adjustment Accuracy Test for NB-IoT UE in Enhanced Coverage | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-170598 | 1683 | - | Addition of the new TC 8.16.53 4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-170629 | 1684 | - | Resubmission of R5-170023 Introduction of RRM Annex I for NB-IoT | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-170713 | 1696 | - | Annex update to RRM spec for LAA | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-170727 | 1698 | - | Introduction of Test case 8.16.63 4DL CA Activation/Deactivation PCell in FDD, unknown SCell | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-170795 | 1706 | - | Uncertainties and Test Tolerances for the Test Cases 9.1.29 and 9.1.30 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-170821 | 1709 | - | Addition of Table H.3.1-2a for E-UTRAN intra frequency measurement configuration for Cat-M1 UE | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-170871 | 1710 | - | (Editorial) Correction of Cell mapping information in 36.521-3 Annex E for RRM TC 9.2.25.1 and 9.2.25.2 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-170878 | 1712 | - | Correction of SRS-Bandwidth for RRM TC 7.1.10 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-170882 | 1714 | - | Correction of eMTC RRM TC 6.2.10 and 6.2.10 prach-StartingSubframe | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-170963 | 1715 | - | Correction of NPDSCH RMC for NB-IoT | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-170977 | 1720 | - | Addition of new test case 9.1.60 FS3 absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal with FDD Pcell | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-170978 | 1721 | - | Addition of new test case 9.1.61 FS3 absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal with TDD Pcell | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-170990 | 1725 | - | Addition of Event triggered reporting on LAA deactivated SCell and E-UTRAN FDD PCell interruption in non-DRX | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-170998 | 1726 | - | Correct test procedures to DC RRM TC 8.23.x | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-170999 | 1727 | - | Update Annex A.2 for DC PCFICH/PDCCH/PHICH parameters | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171000 | 1728 | - | Introduction of new eMTC RRM TC 8.1.31 for category M1 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171001 | 1729 | - | Introduction of new eMTC RRM TC 8.1.32 for category M1 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171002 | 1730 | - | Add new eMTC RRM test cases into Annex E | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171005 | 1732 | - | Correction to NB-IoT and LTE Cell naming used in the tests | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171008 | 1733 | - | Introduction of 4DL CA RRM TC 8.16.51 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171032 | 1739 | - | Adding eMTC RRM test cases 6.1.12-14 and 7.1.14-16 to Annex E | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171051 | 1743 | - | Test Tolerance updating of test cases 9.1.8.1, 9.1.9.1 To include band group FDD_B | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171052 | 1744 | - | Addition of the new TC 9.1.45 4DL PCell in TDD RSRP for E-UTRAN in CA | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171053 | 1745 | - | Addition of the new TC 9.1.48 5DL PCell in FDD RSRP for E-UTRAN in CA | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171054 | 1746 | - | Addition of the new TC 9.1.49 5DL PCell in TDD RSRP for E-UTRAN in CA | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171056 | 1747 | - | Addition of the new TC 9.2.46 4DL PCell in TDD RSRQ for E-UTRAN in CA | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171057 | 1748 | - | Addition of the new TC 9.2.47 5DL PCell in FDD RSRQ for E-UTRAN in CA | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171059 | 1750 | - | Addition of the new TC 9.2.48 5DL PCell in TDD RSRQ for E-UTRAN in CA | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171061 | 1751 | - | Minor correction about RSRP/RSRQ for FDD-TDD CA test cases | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171074 | 1757 | - | Update clause 3A.4 Antenna principles | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171149 | 1766 | - | Test Tolerance: Adding eMTC RRM test case 7.1.12 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171195 | 1773 | - | Removal of editor’s note for TDD FDD 2CA RRM Test cases | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171281 | 1781 | - | Update of Cell configuration mapping for NB-IoT | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171703 | 1734 | 1 | Updated Annex I.4 RRM requirement exception RRM TC 8.22 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171704 | 1775 | 1 | Correction to neighbour cell config for autonomous gaps test cases | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171727 | 1686 | 1 | Update of HD-FDD UE Transmit Timing Accuracy Tests under enhanced coverage | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171728 | 1688 | 1 | Update of HD-FDD Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171729 | 1689 | 1 | Intra-frequency cell reselection under normal coverage for NB-IOT | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171730 | 1741 | 1 | Adding Test case 7.3.60 HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171731 | 1742 | 1 | Adding Test case 7.3.61 HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in extended coverage | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171739 | 1695 | 1 | LAA:Update to common RRM configuration. | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171740 | 1716 | 1 | Addition of LAA SCell activation and deactivation of known SCell with E-UTRA FDD PCell in non-DRX | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171741 | 1749 | 1 | Adding LAA RRM test case 8.26.9 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171742 | 1752 | 1 | Adding LAA RRM test case 8.26.10 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171743 | 1753 | 1 | Adding LAA RRM test case 9.2.51 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171744 | 1754 | 1 | Adding LAA RRM test case 9.2.52 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171746 | 1759 | 1 | Test tolerance addition for TS 36.521-3 Test case 8.22.5 and 8.22.6 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171755 | 1765 | 1 | LAA: Addition of the new test case and test tolerances for FDD intra frequency absolute and relative RSRP accuracies for SCell with frame structure 3 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171759 | 1691 | 1 | Uncertainties and Test Tolerances for TS 36.521-3 Test Cases 4.2.15, 4.2.16, and 4.2.17 and Corresponding Core Specification Updates | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171768 | 1767 | 1 | Test Tolerance CR, adding eMTC RRM test case 7.1.14 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171769 | 1768 | 1 | Test Tolerance CR, adding eMTC RRM test case 7.1.15 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171770 | 1769 | 1 | Test Tolerance CR, adding eMTC RRM test case 7.1.16 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171771 | 1672 | 1 | Test Tolerances Updates in Test Case: 6.2.14: E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage and Other Updates | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171772 | 1673 | 1 | Test Tolerances Updates in Test Case: 6.2.15: E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage and Other Updates | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171773 | 1670 | 1 | Test Tolerances and Uncertainties Updates in Annexes E and F for Test Cases 6.2.13-15, 7.3.50-51 and 7.3.54-55 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171783 | 1774 | 1 | Correction to test parameters and test tolerance for RRM 2CA asymmetric bandwidth test cases | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171799 | 1674 | 1 | New Test Case: E-UTRAN FDD-FDD Intra Frequency Handover for Cat-M1 UEs in CEModeB | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171800 | 1675 | 1 | New Test Case: E-UTRAN HD-FDD Intra Frequency Handover for Cat-M1 UEs in CEModeB | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171801 | 1676 | 1 | New Test Case: E-UTRAN TDD Intra Frequency Handover for Cat-M1 UEs in CEModeB | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171804 | 1690 | 1 | Update to CE mode A RLM test cases | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171805 | 1711 | 1 | New SIB1-BR-r13 and PRACH Config SIB default introduced for RRM TC 6.2.10 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171806 | 1731 | 1 | Update Intra-frequency test cases for Cat-M1 UE in normal and enhanced coverage | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171808 | 1771 | 1 | Correction to OCNG pattern for Cat-M1 FD-FDD and HD-FDD re-establishment TCs | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171810 | 1779 | 1 | eMTC RRM TC 6.2.10 clarification on test procedure | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171811 | 1780 | 1 | Adding eMTC RRM test case 8.1.36 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171812 | 1782 | 1 | Adding eMTC RRM test case 8.1.37 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171854 | 1701 | 1 | Introduction to RRM test case 8.16.57 4DL FDD CA activation and deactivation of known SCell in non-DRX | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171855 | 1702 | 1 | Introduction to RRM test case 8.16.61 4DL FDD CA activation and deactivation of unknown SCell in non-DRX | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171856 | 1705 | 1 | Introduction to RRM test case 9.1.50 5DL FDD RSRP for E-UTRAN in Carrier Aggregation | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171857 | 1777 | 1 | New RRM TC 8.16.55: E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171858 | 1707 | 1 | Introduction of Rel-13 CA RRM new test case 9.1.46 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171859 | 1708 | 1 | Introduction of Rel-13 CA RRM new test case 9.1.47 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171873 | 1758 | 1 | Adding new test case 8.3.7 for IncMon RRM testing | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171874 | 1761 | 1 | Adding new test case 8.4.7 for IncMon RRM testing | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171886 | 1735 | 1 | Update RRM general clause 3 for V2X information | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171891 | 1755 | 1 | Adding new test case 7.3.3_1 for 4 Rx RRM RLM TDD testing | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171892 | 1756 | 1 | Adding new test case 7.3.4_1 for 4 Rx RRM RLM TDD testing | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171899 | 1699 | 1 | Introduction of Test case 8.16.59 4DL CA Activation/Deactivation PCell in FDD, known SCell | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171901 | 1679 | 1 | Update TS 36.521-3 with Addition of LTE Band 48 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171926 | 1738 | 1 | Updates to HD-FDD RLM In-sync tests for category NB1 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171927 | 1717 | 1 | Test tolerance CR, Addition of new test case 8.26.5 E-UTRAN FDD-TDD intra-frequency event triggered reporting in non-DRX under FS3 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171928 | 1718 | 1 | Test tolerance CR, Addition of new test case 8.26.6 E-UTRAN TDD-TDD intra-frequency event triggered reporting in non-DRX under FS3 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171929 | 1719 | 1 | Test tolerance CR, Update Annex F for test tolerance of test cases 8.26.5 and 8.26.6 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171933 | 1671 | 1 | Test Tolerances Updates in Test Case: 6.2.13: E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage and Other Updates | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171960 | 1737 | 1 | Uncertainties and Test Tolerance for DC RRM intra-frequency event triggered reporting test cases | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171964 | 1722 | 1 | Test tolerance CR, Addition of new test case 7.3.62 Radio Link Monitoring Test for In-sync with DRX under EC for Category NB1 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171965 | 1723 | 1 | Test tolerance CR, Addition of new test case 7.3.63 Radio Link Monitoring Test for In-sync with DRX under NC for Category NB1 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171971 | 1700 | 2 | Correction to intra frequency handover test cases for Cat-M1 UEs in CEModeA | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171972 | 1703 | 2 | Uncertainties and Test Tolerances for the test cases 8.1.33, 8.1.34 and 8.1.35 for Cat-M1 UE in CEModeB | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171973 | 1704 | 2 | Uncertainties and Test Tolerances for the Test Cases 8.1.23, 8.1.24, and 8.1.25 for Cat-M1 UE in CEModeA | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171974 | 1736 | 2 | Uncertainties and Test Tolerance for Cat M1 TC8.1.29 and 8.1.30 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171975 | 1740 | 1 | Updating eMTC RRM test cases 6.1.9 - 11 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171976 | 1762 | 2 | Test Tolerance CR, adding eMTC RRM test case 6.1.12 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171977 | 1763 | 2 | Test Tolerance CR, adding eMTC RRM test case 6.1.13 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171978 | 1764 | 2 | Test Tolerance CR, adding eMTC RRM test case 6.1.14 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171979 | 1772 | 2 | Correction to MPDCCH and PDSCH Reference Channels for Cat-M1 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171985 | 1787 | 1 | Correction invalid RI-ConfigIndex in TC 8.16.38 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171986 | 1693 | 2 | Uncertainties and Test Tolerances for RRM TC 9.1.18.2_1, 9.1.19.2_1 and 9.1.24.2_1 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171987 | 1694 | 2 | Uncertainties and Test Tolerances for RRM TC 9.9.1.1_1 and 9.9.2.1_1 | 14.0.1 | 14.1.0 |
| 2017-06 | RAN#76 | R5-172197 | 1793 | - | Update to RRM test cases 8.16.57 and 8.16.61 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172237 | 1799 | - | Update Test Case 5.1.16: E-UTRAN FDD-FDD Intra Frequency Handover for Cat-M1 UEs in CEModeB with Test Tolerance | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172238 | 1800 | - | Update Test Case 5.1.17: E-UTRAN HD-FDD Intra Frequency Handover for Cat-M1 UEs in CEModeB with Test Tolerance | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172239 | 1801 | - | Update Test Case 5.1.18: E-UTRAN TDD Intra Frequency Handover for Cat-M1 UEs in CEModeB | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172240 | 1802 | - | Update Test System Uncertainties and Test Tolerances in Annex F for RRM Requirements of Test Cases 5.1.16, 5.1.17 and 5.1.18 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172256 | 1806 | - | Update of the Message contents about TDD FDD 4/5DL CA RSRP TCs | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172257 | 1807 | - | Update of the Message contents about TDD FDD 4/5DL CA RSRQ TCs | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172273 | 1812 | - | Correction of message contents for RRM eMTC TC 4.2.12 and 4.2.15 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172285 | 1821 | - | Completion of Test cases 8.16.59 and 8.16.63, 4 DL PCell in FDD CA Activation and Deactivation of Known / Unknown SCell in Non-DRX. | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172329 | 1825 | - | Update Test Tolerances and Test requirements for TS 36.521-3 Test cases 9.1.7.1_1+9.1.13.1_1 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172331 | 1826 | - | Update Test Tolerances and Test requirements for TS 36.521-3 Test cases 9.1.7.2_1+9.1.13.2_1 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172384 | 1836 | - | Updated minimum conformance requirement to TC4.2.15 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172389 | 1840 | - | Correction to complete eMTC RRM 8.1.x tests | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172390 | 1841 | - | Updated to eMTC RRM TC 8.1.31 and 8.1.32 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172391 | 1842 | - | Add RMC for V2V Sidelink Communication | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172420 | 1846 | - | DC: Addition of Interruption test cases for Dual Connectivity | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172538 | 1849 | - | Reducing test time for eMTC RRM test cases | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172545 | 1855 | - | Test Tolerance CR to update Annex F for LAA test cases 8.26.2, 8.26.4, 9.1.60 and 9.1.61 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172631 | 1866 | - | Correction to RRM 4DL FDD CA Activation and Deactivation test cases | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172660 | 1868 | - | Editorial correction of TDD-FDD 2CA measurement accuracy test cases | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172661 | 1869 | - | Correction to test parameters and test tolerance for RRM 2CA asymmetric bandwidth test case 8.16.22 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172671 | 1871 | - | eMTC RRM: Correction to MPDCCH and PDSCH Reference Measurement Channels | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172688 | 1872 | - | Test Tolerance: Updates of test cases 8.26.9 and 8.26.10 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172704 | 1874 | - | Test Tolerance: Updates of test cases 9.2.51 and 9.2.52 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172733 | 1876 | - | Test requirements addition and other updates for TS 36.521-3 Test case 8.22.11 and 8.22.12 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172735 | 1877 | - | Update of TS 36.521-3 Test case 8.22.5 and 8.22.6 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172748 | 1880 | - | Updates of test cases 7.1.10, 7.1.11 and 7.1.12 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172750 | 1882 | - | Test Tolerance updating of test cases 9.1.8.2, 9.1.9.2 To include band group FDD_B | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172753 | 1883 | - | Adding RLM test cases for 4Rx UEs | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172754 | 1884 | - | Updates of test cases in Clause 4 to cater for 4Rx testing. | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172755 | 1885 | - | Addition of band group table for NB-IoT operating bands | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172757 | 1886 | - | Addition of two IncMon test cases 8.3.8 and 8.3.9 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173055 | 1823 | 1 | Introducing transition period for legacy 2Rx test cases impacted by Rel-13 4Rx WI | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173056 | 1791 | 1 | LAA: Test tolerance analysis for RSRP accuracy and addition of TDD-FS3 RSRP accuracy test case | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173057 | 1829 | 1 | New RRM LAA TCs 9.11.1 and 9.11.2 including Test Tolerances | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173061 | 1857 | 2 | Test tolerance updates for the test cases 7.1.17 and 7.1.18 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173066 | 1837 | 2 | Introduction of test cases 7.2.10 in CEModeB with Test tolerance | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173067 | 1838 | 2 | Introduction of test cases 7.2.11 in CEModeB with Test tolerance | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173068 | 1839 | 2 | Introduction of test cases 7.2.12 in CEModeB with Test tolerance | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173225 | 1792 | 1 | Correction on Annex J | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173226 | 1817 | 1 | Correct TC title RRM 5DL CA testcases | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173229 | 1861 | 1 | Addition of SIB exceptions for RLM for UE category NB1 in Annex H | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173231 | 1875 | 1 | Test requirements addition and other updates for TS 36.521-3 Test case 8.22.7 and 8.22.8 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173233 | 1833 | 1 | Updated to RRM 4DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX test cases and Test tolerance | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173234 | 1834 | 1 | Introduction of 4DL CA RRM TC 8.16.52 with uncertainties and test tolerance | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173235 | 1863 | 1 | Introduction of 4DL CA RRM TC 8.16.54 with Test tolerance | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173237 | 1789 | 1 | LAA: Test Tolerance analysis and test update of LAA Scell activation and deactivation | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173238 | 1852 | 1 | Test Tolerance CR, addition of LAA test cases 8.26.2 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173239 | 1819 | 1 | Introduction of Test case 8.26.7 E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal, including Test Tolerances | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173240 | 1820 | 1 | Introduction of Test case 8.26.8 E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal, including Test Tolerances | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173242 | 1854 | 1 | Test Tolerance CR of LAA test cases 9.1..60 and 9.1.61 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173244 | 1853 | 1 | Test Tolerance CR, addition of LAA test cases 8.26.4 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173245 | 1790 | 1 | LAA: Test Tolerance analysis and test update of LAA Scell event reporting for deactivated cell | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173248 | 1856 | 1 | Test tolerance updates for the test cases 6.1.15 and 6.1.16 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173252 | 1858 | 1 | Test tolerance updates for the test cases 7.2.9 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173254 | 1859 | 1 | Test tolerance updates for the test cases 7.3.62 and 7.3.63 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173255 | 1860 | 1 | Test tolerance updates in Annex F for the test cases 6.1.15, 6.1.16, 7.1.17, 7.1.18, 7.2.9, 7.3.62 and 7.3.63 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173257 | 1888 | 1 | Test Tolerances for NB-IoT In-Sync RLM test cases 7.3.64 and 7.3.65 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173258 | 1808 | 1 | Addition of SIB and default RRC message content exception for eMTC. Part 1: annex H | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173260 | 1794 | 1 | Test Tolerance addition and update of test case E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173261 | 1795 | 1 | Test Tolerance addition and update of test case E-UTRAN TDD Radio Link Monitoring Test for In-sync for Cat-M1 UE in CEMode A | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173262 | 1796 | 1 | Test Tolerance addition and update of test case E-UTRAN TDD Radio Link Monitoring Test for Out-of-Sync in DRX for UE Category M1 Configured in CEMode A | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173263 | 1797 | 1 | Test Tolerance addition and update of test case E-UTRAN TDD Radio Link Monitoring Test for In-Sync in DRX for UE Category M1 Configured in CEMode A | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173264 | 1798 | 1 | Update Annex E and Annex F Test Tolerance for TC7.3.56,7.3.57,7.3.58,7.3.59 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173265 | 1803 | 1 | Update Test Case 6.2.10: E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage with Test Tolerance Values and Other Changes | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173266 | 1804 | 1 | Update Test Case 6.2.11: E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage with Test Tolerance Values and Other Changes | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173267 | 1805 | 1 | Update Test Tolerance FFS in Test Cases 6.2.13, 6.2.14, 6.2.15 and Annex F 3.2 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173272 | 1827 | 1 | Update Test Tolerances and Test requirements for TS 36.521-3 Test cases 9.1.8.1_1+9.1.9.1_1 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173273 | 1828 | 1 | Update Test Tolerances and Test requirements for TS 36.521-3 Test cases 9.1.10.1_1+9.1.11.1_1 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173275 | 1843 | 1 | Updated to NB-IoT RRM TC 4.2.18 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173283 | 1818 | 1 | Correct TC title RRM LAA testcases | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173284 | 1830 | 1 | New RRM LAA TCs 9.12.1 and 9.12.2 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173285 | 1831 | 1 | LAA Annex E cell mapping for RRM TC 9.11.1, 9.11.2, 9.12.1 and 9.12.2 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173286 | 1878 | 1 | Updates of LAA test cases 8.26.9 and 8.26.10 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173287 | 1879 | 1 | Updates of LAA test cases 9.2.51 and 9.2.52 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173309 | 1835 | 1 | Add new Rel-13 CA 4DL RRM test cases into Annex E | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173310 | 1844 | 1 | Introduction of new Test Case 8.16.67 "5 DL PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX" | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173311 | 1845 | 1 | Introduction of new Test Case 8.16.69 "E-UTRAN FDD 5DL CA activation and deactivation of unknown SCell in non-DRX" | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173312 | 1850 | 1 | Introduction to RRM test case 8.16.58 E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173313 | 1851 | 1 | Introduction to RRM test case 8.16.60 E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173314 | 1865 | 1 | Correction to RRM 4DL FDD CA Event Triggered Reporting test cases | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173319 | 1809 | 1 | Corrections RRM eMTC 6.2.x TCs | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173320 | 1810 | 1 | Correction of message contents for RRM eMTC HO TCs (5.1.13 and 5.1.15) | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173321 | 1813 | 1 | Correction of SRS BW and message contents for eMTC 7.1.X and 7.2.6 TCs | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173322 | 1814 | 1 | Correction to OCNG pattern for all eMTC RRM TCs | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173323 | 1824 | 1 | Correct reference of connection diagram in eMTC MISO RRM tests | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173325 | 1867 | 1 | Clarification of Inter-band configuration for TDD Band 34 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173328 | 1887 | 1 | New test cases: Addition of TC7.4.3 E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous DC | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173334 | 1873 | 1 | E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173335 | 1890 | 1 | E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173398 | 1848 | 1 | Introduction of 5DL TDD RSRP for E-UTRAN in Carrier Aggregation | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173407 | 1816 | 1 | NCell configuration mapping correction for RRM NB-IOT TC 4.2.18 | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173425 | 1881 | 1 | Updates of test cases 7.3.60 and 7.3.61 | 14.1.0 | 14.2.0 |
| 2017-09 | RAN#77 | R5-173557 | 1892 | - | Correction of Timing Advance Adjustment Accuracy Test for Cat-M1 UE | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173558 | 1893 | - | Correction of Annex H for Chapter 7 eMTC | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173560 | 1895 | - | Editorial Change to add CA Class B to cell config mapping | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173561 | 1896 | - | Editorial Change to eMTC event triggered 8.1.24 TC | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173563 | 1898 | - | Editorial Changes to eMTC UL Timing TCs | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173564 | 1899 | - | Correction of Cell 2 SNR for 7.3.38 and 7.3.38_1 (Editorial Change) | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173614 | 1904 | - | Introduction of Test case 8.16.64 4DL Pcell in TDD CA activation and deactivation of unknown SCell in non-DRX | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173750 | 1906 | - | Update Test Tolerances and Test requirements for TS 36.521-3 Test cases 8.16.55+8.16.56 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173754 | 1908 | - | Update Test Tolerances and Test requirements for TS 36.521-3 Test cases 9.1.14.1_1+9.1.15.1_1 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173762 | 1914 | - | Editorial Change of 7.4 Dual Connectivity clause to correct heading level | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173779 | 1915 | - | Editorial correction to 4DL CA activation and deactivation test case 8.16.61 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173780 | 1916 | - | Correction to cell configuration mapping for 4DL CA RRM testing | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173781 | 1917 | - | Editorial correction to references to Annex E | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173790 | 1920 | - | Correction to Initial condition for NB-IoT RRM test cases | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173791 | 1921 | - | Editorial correction to NB-IoT RLM test cases | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173799 | 1924 | - | Correction to intra frequency case for Cat M1 UE in normal coverage including test tolerance update | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173802 | 1927 | - | Editorial correction to UE Cat M1 RSRP Test Cases | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173807 | 1928 | - | Editorial correction to handover test cases | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173812 | 1929 | - | Test Tolerances, Updates of test cases 7.3.60 and 7.3.61 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173826 | 1930 | - | Test Tolerances, Updates of DC test cases 7.3.44 - 47 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173827 | 1931 | - | Editorial, removal of editor’s notes in DC test cases 7.3.41 - 43 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173828 | 1932 | - | Test Tolerances, Updates of 4Rx RLM test cases | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173934 | 1943 | - | Addition of NB-IoT test case 6.2.16 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173935 | 1944 | - | Addition of NB-IoT test case 6.2.17 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174064 | 1947 | - | Completion of RRM test case 8.16.58 E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174066 | 1948 | - | Completion of RRM test case 8.16.60 E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174131 | 1959 | - | Update to RRM test case 9.1.50 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174140 | 1961 | - | Introduction of 5DL CA RRM TC 8.16.65 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174141 | 1962 | - | Introduction of 5DL CA RRM TC 8.16.66 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174143 | 1964 | - | Introduction of 5DL CA RRM TC 8.16.72 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174146 | 1965 | - | Corrected Test Applicability to 3DL CA tests required event A6 [TEI11] | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174147 | 1966 | - | Corrected Test Applicability to 3DL CA tests required event A6 [TEI12] | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174161 | 1967 | - | Addition of SIB and default RRC message content exception for remaining eMTC tests - Chapter 4 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174162 | 1968 | - | Addition of SIB and default RRC message content exception for remaining eMTC tests - Chapter 6 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174165 | 1971 | - | Completion of LAA TCs 8.26.x | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174166 | 1972 | - | Completion of LAA TC 9.1.60 and 9.1.61 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174167 | 1973 | - | Editorial Change to TC 9.11.1 and 2 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174168 | 1974 | - | Editorial Changes to TC 9.1.55 and 56 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174169 | 1975 | - | Annex E updates for LAA | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174185 | 1976 | - | Update and inclusion of Test Tolerances for Cat M1 test cases A.6.2.10/11/12 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174273 | 1982 | - | Updates to 36.521-3 related to intra/inter-frequency SI acquisition for HO (AP#75.22) | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174321 | 1985 | - | Addition of SIB and default RRC message content exception for remaining eMTC tests - Chapter 8 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174322 | 1986 | - | Addition of SIB and default RRC message content exception for remaining eMTC tests - Chapter 9 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174323 | 1987 | - | New RMCs for eMTC DRX test cases - Annex | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174324 | 1988 | - | New RMCs for eMTC DRX test cases | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174332 | 1990 | - | Editorial Change to update RMC for V2V Sidelink Communication | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174368 | 1997 | - | Addition of new DC test cases 8.23.7 - 9 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175013 | 1902 | 1 | Introduction of Test case 8.16.56 4DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175031 | 1980 | 1 | Test tolerance and test case update for Interruption test cases with Dual Connectivity | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175033 | 1963 | 1 | Introduction of 5DL CA RRM TC 8.16.71 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175037 | 1939 | 1 | Test tolerance, addition of NB-IoT test case 7.3.67 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175038 | 1938 | 1 | Test tolerance, addition of NB-IoT test case 7.3.66 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175039 | 1940 | 1 | Test tolerance, addition of cell mapping and test uncertainty in Annex E and F | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175040 | 1933 | 1 | Test Tolerances, Updates of test cases 7.1.14, 7.1.15 and 7.1.16 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175041 | 1934 | 1 | Update Undefined Parameters and Add Test Tolerance Values for Test Case 7.3.50 E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175042 | 1935 | 1 | Update Undefined Parameters and Add Test Tolerance Values for Test Case 7.3.51 E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE category M1 configured in CEMode A | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175043 | 1936 | 1 | Resubmit New Test Case: 7.3.54: E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE Category M1 Configured in CEMode A with Parameters Updated and Test Tolerances Added | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175044 | 1937 | 1 | Update Undefined Parameters and Add Test Tolerance Values for Test Case 7.3.55 E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE category M1 configured in CEMode A | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175045 | 1907 | 1 | Update Test Tolerances and Test requirements for TS 36.521-3 Test case 9.1.5.2_1 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175046 | 1945 | 1 | Update to the LAA test cases 8.26.3 and 8.26.4 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175047 | 1958 | 1 | LAA: Correction to event reporting test cases | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175053 | 1994 | 1 | Updates to clause 6 RRM test cases to cater for 4Rx only UEs | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175054 | 1993 | 1 | Updates to clause 5 RRM test cases to cater for 4Rx only UEs | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175061 | 1910 | 1 | Correction of the message contents of RRM UL CA TCs | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175062 | 1983 | 1 | Annex E update for 3-DL CA deactivated SCELL measurement | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175069 | 1894 | 1 | Correction of message contents for eMTC HO 5.1.13 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175070 | 1925 | 1 | Correction to RLM test cases for Cat M1 in CE mode A | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175071 | 1984 | 1 | Addition of SIB and default RRC message content exception for remaining eMTC tests - Chapter 7 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175097 | 1905 | 1 | Update Test Tolerances and Test requirements for TS 36.521-3 Test case 9.1.5.1_1 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175098 | 1946 | 1 | Editorial: Addition of Conditions for V2X | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175109 | 1991 | 1 | Update RRM general clause 3 for V2X information | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175132 | 1903 | 1 | Introduction of Test case 8.16.62 4DL TDD CA activation and deactivation of unknown SCell in non-DRX | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175133 | 1960 | 1 | Added new Cell configuration mapping table for 5DL RRM test cases into Annex E | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175134 | 1977 | 1 | Introduction of new RRM FDD 5DL CA TC 8.16.75 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175135 | 1978 | 1 | Introduction of new RRM FDD 5DL CA TC 8.16.77 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175141 | 1995 | 1 | Updates to clause 8 RRM test cases to cater for 4Rx only UEs | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175149 | 1911 | 1 | Alignment of Antenna Config for NB-IOT Test Cases and addition of SIB1 In-Band Mode exceptions | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175150 | 1912 | 1 | Alignment of Annex E for NB-IOT and correction of Cell IDs | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175151 | 1913 | 1 | Completion of RRM NB-IOT TC 6.1.15 and 16 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175153 | 1919 | 1 | Correction to cell configuration mapping for NB-IoT | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175154 | 1922 | 1 | Correction to HD-FDD UE Timing Advance Adjustment Accuracy test case 7.2.9 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175155 | 1941 | 1 | Addition of NB-IoT test case 4.2.19 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175157 | 1969 | 1 | Corrections to TC 4.2.18 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175158 | 1970 | 1 | Alignment of NOCGN patterns with core spec | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175162 | 1979 | 1 | Test Tolerances for TCs 8.1.26, 8.1.27 & 8.1.28 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175163 | 1992 | 1 | Updates of TCs 8.1.36 - 37 adding TCs 8.1.38-39 and 8.2.9-10 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175173 | 1999 | 1 | Add New TC of 8.16.73 for E-UTRAN FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175209 | 1918 | 2 | Editorial correction to NB-IoT RLM test cases 7.3.60 and 7.3.61 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175212 | 1996 | 2 | Updates to clause 9 RRM test cases to cater for 4Rx only UEs | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175213 | 1900 | 1 | Update Annex F Test Tolerances for TS 36.521-3 Test cases 9.12.1 and 9.12.2 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175214 | 1901 | 1 | Update Test Tolerances for TS 36.521-3 Test cases 9.12.1 and 9.12.2 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175216 | 1942 | 2 | Addition of NB-IoT test case 4.2.24 | 14.2.0 | 14.3.0 |
| 2017-12 | RAN#78 | R5-176076 | 2000 | - | Introduction of E-UTRAN FDD - FDD cell re-selection intra frequency case for UE configured with highSpeedEnhancedMeasFlag | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176077 | 2001 | - | Introduction of E-UTRAN TDD - TDD cell re-selection intra frequency case for UE configured with highSpeedEnhancedMeasFlag | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176078 | 2002 | - | Introduction of E-UTRAN TDD-TDD intra-frequency event triggered reporting for UE configured with highSpeedEnhancedMeasFlag in synchronous cells in 8.2.11 | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176091 | 2003 | - | Editorial - Remove Editor Note in 9.12 | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176092 | 2004 | - | Editorial - Correction to 8.22.1 | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176093 | 2005 | - | Editorial - Correction to 7.1.7 | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176094 | 2006 | - | Correction of TC applicability in 36.521-3 for chapter 8 RRM eMTC | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176095 | 2007 | - | Correction of Connection Diagram for eMTC reselection tests | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176096 | 2008 | - | Completion of eMTC 6.2.x series | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176097 | 2009 | - | Correction of TC 8.26.1 test procedure to have only 1 SCC | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176099 | 2011 | - | Changes to Annex E for NB-IOT | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176100 | 2012 | - | Correction of Connection Diagram for NB-IOT reselection tests | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176101 | 2013 | - | Correction of Connection Diagram for NB-IOT UL timing and RLM tests | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176237 | 2019 | - | Introduction of New TC 8.16.70 5DL TDD CA activation and deactivation of unknown SCell in non-DRX | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176238 | 2020 | - | Introduction of New TC 8.16.74 5DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176239 | 2021 | - | Update to applicability of the RRM test case 8.16.57 and 8.16.61 for 4DL CA | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176309 | 2029 | - | Added RMC for V2X Sidelink Communication | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176442 | 2034 | - | Test tolerance, update of 4.2.19 and 4.2.24 to add test tolerances | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176446 | 2036 | - | Test tolerance, Addition of test case 8.4.6, TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0 | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176489 | 2039 | - | Correction to 4DL FDD-TDD CA RSRQ Test 9.2.45 | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176490 | 2040 | - | Correction to 4CA and 5CA RSRP accuracy tests | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176512 | 2044 | - | Updates to clause 8 single carrier test cases for 4Rx only UEs | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176513 | 2045 | - | Updates to clause 9 single carrier test cases for 4Rx only UEs | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176514 | 2046 | - | Editorial correction to Test Parameters for FS3 Channel Occupancy tests | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176700 | 2052 | - | Changes to RRM Chapter 7 4Rx tests | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176771 | 2054 | - | Alignment of NOCGN pattern 1 with TS 36.133 | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176862 | 2061 | - | Alignment of 4DL and 5DL RRM tes case titles | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177088 | 2022 | 1 | Update NB-IoT RLM Out-of-sync Test cases | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177105 | 2016 | 2 | Update Test Tolerances and Test requirements for TS 36.521-3 Test cases 9.1.46+9.1.47 | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177115 | 2048 | 2 | Correction to 7.1.17 | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177116 | 2049 | 1 | Correction to 7.1.18 | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177308 | 2014 | 1 | TC applicability an completion of RRM 4DL CA chapter 8 test cases | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177310 | 2015 | 1 | Update Test Tolerances and Test requirements for TS 36.521-3 Test cases 9.1.44+9.1.45 | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177312 | 2017 | 1 | Update Test Tolerances and Test requirements for TS 36.521-3 Test cases 9.2.45+9.2.46 | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177314 | 2032 | 1 | eLAA: Addition of 3-DL event reporting test case for deactivated Scell | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177315 | 2059 | 1 | eLAA: Addition of TDD FS3 event triggered reporting | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177322 | 2056 | 1 | Correction to FDD_B requirements in 9.1.10.1 and 9.1.14.1 | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177332 | 2051 | 1 | Changes to RRM Chapter 4 4Rx tests | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177350 | 2062 | - | Add New TC of 8.16.68 for 5DL Pcell in TDD CA activation and deactivation of unknown SCell in non-DRX | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177358 | 2028 | 1 | Updated to RMC for V2V Sidelink Communication | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177359 | 2030 | 1 | Introduction of RRM TC 11.1 | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177360 | 2031 | 1 | Introduction of RRM TC 11.2 | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177372 | 2057 | 1 | Addition of new TC for V2X_RRM | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177382 | 2010 | 1 | Completion of TC 7.3.49 | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177397 | 2024 | 1 | Introduction of RRM TC 4.2.20 | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177398 | 2025 | 1 | Introduction of RRM TC 4.2.21 | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177399 | 2026 | 1 | Introduction of RRM TC 5.1.19 | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177400 | 2027 | 1 | Introduction of RRM TC 5.1.20 | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177432 | 2058 | 1 | eLAA: Addition of FDD FS3 event triggered reporting | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177448 | 2055 | 1 | Correction of Antenna Configuration in some RadioLinkMonitoring 4Rx test cases | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177450 | 2037 | 1 | Update RRM test cases 9.1.4.1_1 and 9.1.4.2_1 to add test configurations for TDD UL/DL configuration 0 | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177451 | 2038 | 1 | Update RRM test cases 9.2.4.1_1 and 9.2.4.2_1 to add test configurations for TDD UL/DL configuration 0 | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177456 | 2023 | 1 | Update NB-IoT RLM In-sync Test cases | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177457 | 2060 | 2 | Cat1bis new test case: RSRP Measurement accuracy test case for CAT1bis UE | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176679 | 2047 | - | Band 71 information to E-UTRA band group in 36.521-3 | 14.4.0 | 15.0.0 |
| 2018-03 | RAN#79 | R5-180336 | 2069 | - | Band 72 information to E-UTRA band group in 36.521-3 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180337 | 2070 | - | Band 68 information to E-UTRA band group in 36.521-3 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180397 | 2072 | - | Test tolerance, Addition of new test case 7.6.1 to 36.521-3 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180398 | 2073 | - | Test tolerance, Addition of new test case 7.6.2 to 36.521-3 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180399 | 2074 | - | Test tolerance, Addition of cell mapping and test tolerance in 36.521-3 Annex E and F for RRM test cases 7.6.1 and 7.6.2 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180411 | 2077 | - | Test tolerance, Update of Annex F to add TT information for 6.2.16 and 6.2.17. | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180589 | 2080 | - | Correct RRM LAA test applicability in 36521-3 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180591 | 2082 | - | [Editorial] Correction of cell SNR for 7.3.3 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180593 | 2084 | - | [Editorial] Correction of prach-StartingSubframe-r13 for 6.2.10 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180594 | 2085 | - | Correction of 7.3.48 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180595 | 2086 | - | Correction of 7.3.49 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180596 | 2087 | - | Correct test equipment connections for RRM 2CC tests - Chapter 8 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180597 | 2088 | - | Correct test equipment connections for RRM 2CC tests - Chapter 8 Inter-Frequency | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180598 | 2089 | - | Correct test equipment connections for RRM 2CC tests - Chapter 9 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180599 | 2090 | - | Correct test equipment connections for RRM 2CC tests - Chapter 9 RSRQ | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180602 | 2093 | - | Correct test equipment connections for RRM 3CC tests - Chapter 9 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180603 | 2094 | - | Correct test equipment connections for RRM 3CC tests - Chapter 9 RSRQ | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180606 | 2097 | - | Correct test equipment connections for RRM 4CC tests - Chapter 9 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180607 | 2098 | - | Correct test equipment connections for RRM 4CC tests - Chapter 9 RSRQ | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180744 | 2100 | - | Updated to Cat-1bis RRM TC 4.2.20 and 4.2.21 and Test Tolerance | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180745 | 2101 | - | Updated to Cat-1bis RRM TC 5.1.19 and 5.1.20 and Test Tolerance | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180912 | 2110 | - | Cat1bis: Addition of new CAT1-bis Intra-Frequency Event triggered reporting TC | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180937 | 2117 | - | Editorial correction to a incorrect reference in 8.16.59 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180952 | 2118 | - | Correction to clause 7.3 RRM 4Rx test cases | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180955 | 2121 | - | Correction to channel bandwidth to be tested for LAA RRM test cases | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180956 | 2122 | - | Correction to successful report rate threshold for LAA channel occupancy tests | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180957 | 2123 | - | Correction to LAA CSI-RSRP measurement tests | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180959 | 2125 | - | Correction to LAA RSSI measurement and channel occupancy tests | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181004 | 2127 | - | Introduction to the new 5DL CA test case 8.16.76 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181087 | 2129 | - | Update of Incmon RRM test cases 8.3.7, 8.3.8, 8.3.9 and 8.4.7 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181104 | 2130 | - | Update of NPRACH Test cases 6.2.16 and 6.2.17 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181344 | 2124 | 1 | Correction to LAA RSRP/RSRQ measurement tests | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181358 | 2075 | 2 | Test tolerance, Update of test case 6.2.16 to add TT information. | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181359 | 2076 | 2 | Test tolerance, Update of test case 6.2.17 to add TT information. | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181363 | 2126 | 2 | Correction to CatNB1 HD-FDD Transmit Timing Accuracy Tests | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181514 | 2078 | 1 | Insertion of test tolerance statements for HST test cases 4.2.22&4.2.23&8.2.11 in Annex F | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181515 | 2079 | 1 | Update the HST test cases in line with the test tolerance analysis in 4.2.22,4.2.23 and 8.2.11 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181528 | 2083 | 1 | Cell mapping correction for 4DL CA Tests | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181541 | 2114 | 1 | eLAA: Addition of 3-DL event reporting test case for deactivated Scell | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181556 | 2120 | 1 | Correction to test applicability of LAA RRM test cases | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181589 | 2107 | 1 | Cat1bis RRM RSRQ Annex E | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181590 | 2108 | 1 | Cat1bis RRM RSRQ Annex F | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181591 | 2109 | 1 | Addition of Cat1bis CGI Reading tests | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181592 | 2111 | 1 | Cat1bis: Addition of new CAT1-bis RSRQ Measurement accuracy test case | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181593 | 2112 | 1 | Cat1bis: Addition of new CAT1-bis Transmit timing accuracy test case | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181594 | 2113 | 1 | Cat1bis: Addition of new CAT1-bis Timing advance accuracy test case | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181595 | 2116 | 1 | Updates to Cat1bis RSRP Measurement accuracy test cases | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181598 | 2115 | 1 | Cat1bis: Addition of new radio link monitoring test cases for CAT1-bis UE | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181611 | 2128 | 1 | Addition of the Band 74 information into TS 36.521-3 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181635 | 2131 | 1 | Update of V2V RRM Test Cases | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181655 | 2091 | 1 | Correct test equipment connections for RRM 3CC tests - Chapter 8 unknown Scell | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181656 | 2092 | 1 | Correct test equipment connections for RRM 3CC tests - Chapter 8 known Scell | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181657 | 2095 | 1 | Correct test equipment connections for RRM 4CC tests - Chapter 8 unknown Scell | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181658 | 2096 | 1 | Correct test equipment connections for RRM 4CC tests - Chapter 8 known Scell | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181659 | 2119 | 1 | Correction to LAA RRM tests 8.26.x | 15.0.0 | 15.1.0 |
| 2018-06 | RAN#80 | R5-182193 | 2132 |  | Completion of RRM TC 9.2.4.1_1 and 9.2.4.2_1 | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182194 | 2133 |  | TC 9.2.4.1_1 and 9.2.4.2_1 Annex F | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182196 | 2135 |  | Editorial correction of RRM TC 8.16.55 | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182198 | 2137 |  | Completion of RRM TC 8.16.68 | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182200 | 2139 |  | Applicability for RRM TC 8.16.70 | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182201 | 2140 |  | Corrections to RRM TC 8.16.71 | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182202 | 2141 |  | Completion of RRM TC 8.16.73 | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182205 | 2144 |  | Editorial correction of RRM TC 9.1.46 | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182206 | 2145 |  | Completion of RRM TC 9.1.48 | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182207 | 2146 |  | Completion of RRM TC 9.1.49 | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182208 | 2147 |  | Completion of RRM TC 9.1.50 | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182209 | 2148 |  | Completion of RRM TC 9.1.51 | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182211 | 2150 |  | Corrections to Annex F for 5DL CA RRM tests | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182277 | 2167 |  | Annex E RRM Cat1bis 6.2.x and 6.3.x tests | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182278 | 2168 |  | Annex F RRM Cat1bis 6.2.x and 6.3.x tests | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182719 | 2182 |  | Introduction of TC 8.16.78 5DL TDD CA activation and deactivation of know SCell in non-DRX | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182728 | 2183 |  | Correction to applicability of LAA clause 9 RRM test cases | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182740 | 2186 |  | Correction to 5DL CA TDD-FDD RSRQ tests | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182754 | 2190 |  | Correction to FS3 event trigger reporting tests | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182755 | 2191 |  | Correction to FS3 channel occupancy tests | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182764 | 2192 |  | Correction connection diagram used for Cat-1bis test cases | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182766 | 2193 |  | Adding MPDCCH information for category M2 | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182767 | 2194 |  | Adding OCNG patter for V2X Sidelink test cases | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182787 | 2195 |  | Addition of Cell configuration Mapping for HST | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182895 | 2199 |  | Editorial correction to CAT-M1 RLM test cases | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183032 | 2204 |  | Updates of Band Group B (Band 74) information about RSRP TCs | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183262 | 2164 | 1 | Test tolerance, Addition of eNB-IoT test case 6.2.18 | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183716 | 2151 | 1 | Update of test case 11.1 and 11.2 to add test tolerance information | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183719 | 2156 | 1 | Introduction of RRM TC 5.1.3_2 with Test Tolerance | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183720 | 2157 | 1 | Introduction of RRM TC 5.1.4_2 with Test Tolerance | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183721 | 2158 | 1 | Introduction of RRM TC 5.1.7_2 with Test Tolerance | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183722 | 2159 | 1 | Introduction of RRM TC 5.1.8_2 with Test Tolerance | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183723 | 2160 | 1 | Introduction of RRM TC 5.1.5_2 with Test Tolerance | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183724 | 2161 | 1 | Introduction of RRM TC 5.1.6_2 with Test Tolerance | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183725 | 2163 | 1 | Insertion of test tolerance statements for Cat-1bis RRM test cases in Annex F | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183726 | 2176 | 1 | Test Tolerance: Updates to Annex F and other details for Cat1bis Tx Timing and RLM test cases | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183729 | 2180 | 1 | Correction to NB-IoT Tx Timing Test cases 7.1.17 and 7.1.18 | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183730 | 2179 | 1 | Add 5MHz eCell Channel BW scenario to NB-IoT Test cases 4.2.18, 6.1.15 and 6.1.16 | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183738 | 2171 | 1 | Update and correction of message content for 5DL CA RRM test case 8.16.67. | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183739 | 2138 | 1 | Completion of RRM TC 8.16.69 | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183740 | 2142 | 1 | Completion of RRM TC 8.16.75 | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183741 | 2174 | 1 | Update and correction of erroneous message content for 5DL CA RRM test cases 8.16.77 "5DL FDD CA activation and deactivation of known SCell in non-DRX". | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183742 | 2185 | 1 | Correction to 5DL CA RRM Test Cases | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183743 | 2149 | 1 | Corrections to Annex E for 5DL CA RRM tests | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183756 | 2187 | 1 | Correction of connection figure for RRM 2Rx 2CC tests without neighbour cells | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183757 | 2188 | 1 | Correction of connection figure for RRM 2Rx 3CC/4CC tests without neighbour cells | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183758 | 2189 | 1 | Correction to FDD-FS3 Activation and Deactivation test cases | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183771 | 2177 | 1 | Update to RRM test case 8.16.70 and 8.16.74 for 5DL CA | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183780 | 2205 |  | Clarify frame structure in 4Rx test principles clause of RRM spec | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183781 | 2206 |  | Introduction of RRM TC for E-UTRAN FDD-FDD Inter frequency case for UE Category 1bis with Test Tolerance | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183782 | 2207 |  | Introduction of RRM TC for E-UTRAN FDD-TDD Inter frequency case for UE Category 1bis with Test Tolerance | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183783 | 2208 |  | Introduction of RRM TC for E-UTRAN TDD-FDD Inter frequency case for UE Category 1bis with Test Tolerance | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183784 | 2209 |  | Introduction of RRM TC for E-UTRAN TDD-TDD Inter frequency case for UE Category 1bis with Test Tolerance | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183799 | 2197 | 1 | update of V2V RRM TC | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183807 | 2162 | 1 | Introduction of Cat 1bis RRM test cases to Annex E | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183810 | 2181 | 1 | V2X: Update to timing accuracy test cases | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183811 | 2196 | 1 | Introduction of V2X RRM TC12.4 | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183827 | 2165 | 1 | New RRM Cat1bis 6.2.x tests | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183828 | 2166 | 1 | New RRM Cat1bis 6.3.x tests | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183829 | 2200 | 1 | Updates to Cat1bis Chapter 7 - RLM TC Annex E | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183830 | 2201 | 1 | Addition of new CAT1bis Ch8 RRM test cases | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183865 | 2178 | 1 | Correction to IncMon RRM Test Cases 8.3.7, 8.3.8, 8.3.9, 8.4.7 | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183866 | 2203 | 1 | Updates of Band Group B (Band 74) information about RSRQ TCs | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183883 | 2170 | 1 | High_Speed_Test: Addition of Intra-frequency event reporting test case | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183899 | 2134 | 1 | Corrections to 9.1.10.1 and 2 for band FDD_B | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183830 | 2201 | 1 | implementation of missing new test cases of R5-183830 | 15.2.0 | 15.2.1 |
| 2018-09 | RAN#81 | R5-184040 | 2213 | - | Deletion of Conditions for GNSS Reliability Requirements for V2X from Annex I.6 | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184102 | 2216 | - | Editorial corrections 8.16.73 | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184105 | 2219 | - | Editorial corrections 7.3.49 | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184106 | 2220 | - | RMC correction 8.1.12_1 (core spec alignment) | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184107 | 2221 | - | New generic CA RSRP test cases | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184108 | 2222 | - | New generic CA RSRQ test cases | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184129 | 2223 | - | Editorial Correction to note 7 in Table E-1 | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184140 | 2224 | - | Removal of technical content in 36.521-3 v14.4.0 and substitution with pointer to the next Release | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184151 | 2226 | - | Test tolerance update of eHST FDD event reporting test case | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184203 | 2233 | - | Test Tolerance: Updates to Cat1bis chapter 8 inter-frequency event triggered reporting tests | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184205 | 2234 | - | Test Tolerance: Updates to Cat1bis Event triggered reporting tests under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184207 | 2235 | - | Test Tolerance: Updates to Cat1bis Event triggered reporting tests with fading, Annex E and Annex F | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184209 | 2236 | - | Test Tolerance: Updates to Cat1bis intra-frequency Event triggered reporting tests, Annex E and Annex F | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184255 | 2238 | - | Editorial correction to Reference NPRACH Configuration | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184256 | 2239 | - | Correction to 5CA FDD-TDD Chapter 8 Tests | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184258 | 2241 | - | Editorial correction to LAA Listen-before-talk model | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184259 | 2242 | - | Correction to connection figure for 4 DL and 5 DL CA Event Triggered Reporting tests | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184260 | 2243 | - | Correction to cell configuration mapping table for 8.16.70 and 8.16.78 | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184261 | 2244 | - | Correction to cell configuration mapping for 8.26.3 | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184262 | 2245 | - | Correction to test procedure for FS3 event triggered reporting tests | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184263 | 2246 | - | Correction to test requirement of FS3 event triggered reporting tests | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184264 | 2247 | - | Correction to minimum conformance requirements for FDD_B1 and FDD_B2 | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184265 | 2248 | - | Editorial correction to Noc for Cell2 on FDD_B1 or FDD_B2 | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184273 | 2249 | - | Addition of V2X new TC 12.2.1 | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184276 | 2252 | - | Update of Annex F for TC 12.2.1, 12.2.2 and 12.4 | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184289 | 2254 | - | Addition of Rel-13 B5C new TC 8.16.47 | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184290 | 2255 | - | Addition of Rel-13 B5C new TC 8.16.48 | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184291 | 2256 | - | Addition of Rel-13 B5C new TC 8.16.49 | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184292 | 2257 | - | Addition of Rel-13 B5C new TC 8.16.50 | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184293 | 2258 | - | Update of Annex E and Annex F to add cell mapping and TT for 8.16.47-50 | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184375 | 2259 | - | Introduction of TC 6.1.1_2 E-UTRAN FDD Intra-frequency RRC Re-establishment for UE Category 1bis | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184376 | 2260 | - | Introduction of TC 6.1.2_2 E-UTRAN FDD Inter-frequency RRC Re-establishment for UE Category 1bis | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184377 | 2261 | - | Introduction of TC 6.1.3_2 E-UTRAN TDD Intra-frequency RRC Re-establishment for UE Category 1bis | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184378 | 2262 | - | Introduction of TC 6.1.4_2 E-UTRAN TDD Inter-frequency RRC Re-establishment for UE Category 1bis | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184379 | 2263 | - | Annex E,F RRM Cat1bis 6.1.x tests | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184421 | 2264 | - | Update to RRM test case 8.16.70 and 8.16.74 for 5DL CA | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184570 | 2265 | - | Corrections for 5MHz Channel BW in 4.2.19, 6.x.y and 7.3.x NB-IoT Test cases | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184667 | 2268 | - | Correction to LAA inter-frequency event-triggered reporting test cases | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184917 | 2284 | - | Correction on 4Rx RLM TC in-sync parameters | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185184 | 2250 | 1 | Addition of V2X new TC 12.2.2 | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185337 | 2237 | 1 | Test Tolerance: Updates to Cat1bis intra-frequency CGI Event triggered reporting tests, Annex E and Annex F | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185342 | 2253 | 1 | Editorial cleaning up of Rel-14 CA 5DL RSRP measurement TC 9.1.51 | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185373 | 2279 | 1 | Update of V2X RRM | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185386 | 2232 | 1 | Completion of Chapter 7 RRM cat 1bis test cases | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185391 | 2227 | 1 | Test Tolerance: Updates to Cat1bis inter frequency RSRP accuracy tests and Annex F | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185392 | 2228 | 1 | Test Tolerance: Updates to Cat1bis inter frequency RSRQ accuracy tests and Annex F | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185393 | 2229 | 1 | Test Tolerance: Updates to Cat1bis intra frequency RSRP accuracy tests and Annex F | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185394 | 2230 | 1 | Test Tolerance: Updates to Cat1bis intra frequency RSRQ accuracy tests and Annex F | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185457 | 2266 | 1 | Addition of RRM IncMon new Test Case 8.4.8 | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185458 | 2267 | 1 | Addition of RRM IncMon new Test Case 8.4.9 | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185459 | 2269 | 1 | Addition of RRM IncMon new Test Case 8.5.8 | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185460 | 2270 | 1 | Addition of RRM IncMon new Test Case 8.6.3 | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185461 | 2271 | 1 | Addition of RRM IncMon new Test Case 8.7.5 | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185462 | 2272 | 1 | Addition of RRM IncMon new Test Case 8.7A.1 | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185463 | 2273 | 1 | Addition of RRM IncMon new Test Cases 4.2.1 and 4.2.11 | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185464 | 2274 | 1 | Addition of RRM IncMon new Test Case 4.3.1.5 | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185465 | 2275 | 1 | Addition of RRM IncMon new Test Case 4.3.2A | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185466 | 2276 | 1 | Addition of RRM IncMon new Test Case 4.3.3A | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185467 | 2277 | 1 | Addition of RRM IncMon new Test Case 4.3.4.4 | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185483 | 2217 | 1 | Editorial corrections 9.1.46 | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185484 | 2218 | 1 | Editorial corrections 9.1.50 | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185485 | 2231 | 1 | Update of RRM IncMon test cases 8.3.7, 8.3.8, 8.3.9, 8.4.7 including test tolerances | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185536 | 2212 | 1 | Correction of GNSS information for V2X tests | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185544 | 2280 | 1 | New TC 8.16.83: 3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185545 | 2281 | 1 | New TC 8.16.84: 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185546 | 2282 | 1 | New TC 8.16.85: 3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185547 | 2283 | 1 | New TC 8.16.86: 3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes | 15.2.1 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185554 | 2225 | 1 | Test tolerance update of V2X uplink timing test cases | 15.2.1 | 15.3.0 |
| 2018-12 | RAN#82 | R5-186478 | 2293 | - | Alignment NPRACH configuration (Editorial) | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-186479 | 2294 | - | Correction for 4.2.19 | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-186480 | 2295 | - | Correction for 4.2.24 Cell ID selection procedure | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-186481 | 2296 | - | Core spec alignment 4.2.24 | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-186483 | 2298 | - | Correction for 7.3.50 and 7.3.51 | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-186484 | 2299 | - | Correction for 8.1.28 | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-186485 | 2300 | - | Removal of reference to H.7.1-2 for RRM 7.3.5x series | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-186486 | 2301 | - | Add band group FDD B1 and B2 to RSRQ tests - Rel8 | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187043 | 2322 | - | Editorial correction to LAA Event Trigger Reporting Test 8.26.7 | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187044 | 2323 | - | Editorial correction to the E-UTRA operating band group for Cat-M1 Clause 9 Tests | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187045 | 2324 | - | Editorial correction to 9.1.10.1 | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187046 | 2325 | - | Correction to Cat-M1 intra-frequency event triggered reporting tests | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187061 | 2328 | - | Correction to 6.1.9 FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187067 | 2331 | - | Correction to FS3 CSI-RSRP accuracy tests | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187334 | 2341 | - | Changing the cell configuration mapping tables to cater for RRM generic duplex mode tests | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187393 | 2342 | - | Addition of Cat. M2 Support to RRM CE tests - Chapter 8 | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187394 | 2343 | - | Addition of Cat. M2 Support to RRM CE tests - Chapter 9 | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187444 | 2344 | - | Introduction of RRM TC 4.2.25 | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187445 | 2345 | - | Introduction of RRM TC 4.2.26 | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187446 | 2346 | - | Introduction of RRM TC 4.2.27 | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187470 | 2347 | - | Correction of TC 8.16.83, 8.16.84, 8.16.85 and 8.16.86 | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187472 | 2348 | - | RRM Generic 3DL CA tests - Annex F | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187850 | 2290 | 1 | Completion of TC 9.1.69 and 9.1.70 | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187851 | 2291 | 1 | Completion of TC 9.2.56 and 9.2.57 | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187853 | 2349 | 1 | Addition of Test Case selection criteria for RRM CA | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187854 | 2350 | 1 | Update on Applicability and Procedure for generic duplex mode RRM test case 8.16.86 | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187855 | 2292 | 1 | Chapter 9 RRM Generic CA tests - Annexes | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187856 | 2327 | 1 | Correction to 3DL CA clause 8 generic duplex mode tests | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187857 | 2336 | 1 | Introduction of 4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187858 | 2337 | 1 | Introduction of 4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187859 | 2338 | 1 | Introduction of 5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187860 | 2339 | 1 | Introduction of 5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187863 | 2332 | 1 | Updated RRM clause 4 tests to cover Cat-M2 requirement | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187869 | 2302 | 1 | Add band group FDD B1 and B2 to RSRQ tests - Rel11 | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187870 | 2303 | 1 | Add band group FDD B1 and B2 to RSRQ tests - Rel12 | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187926 | 2304 | 1 | Correction of RRM IncMon TC 4.2.10 | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187927 | 2305 | 1 | Correction of RRM IncMon TC 4.2.11 | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187928 | 2306 | 1 | Correction of RRM IncMon TC 4.3.1.5 | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187929 | 2307 | 1 | Correction of RRM IncMon TC 4.3.2A | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187930 | 2308 | 1 | Correction of RRM IncMon TC 4.3.3A | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187931 | 2309 | 1 | Correction of RRM IncMon TC 4.3.4.4 | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187932 | 2310 | 1 | Correction of RRM IncMon TC 8.4.8 | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187933 | 2311 | 1 | Correction of RRM IncMon TC 8.4.9 | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187934 | 2312 | 1 | Correction of RRM IncMon TC 8.5.8 | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187935 | 2313 | 1 | Correction of RRM IncMon TC 8.6.3 | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187936 | 2314 | 1 | Correction of RRM IncMon TC 8.7.5 | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187937 | 2315 | 1 | Correction of RRM IncMon TC 8.7A.1 | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187938 | 2316 | 1 | Cell Configuration for IncMon RRM Test Cases | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187939 | 2317 | 1 | New rows in Annex F tables for IncMon RRM Test Cases | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187964 | 2318 | 1 | Update Rel-14 NB-IOT RRM test cases to support category NB2 | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187982 | 2320 | 1 | Addition of B72 for test cases with 5MHz channel bandwidth | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187985 | 2333 | 1 | Updated RRM clause 5 tests to cover Cat-M2 requirement | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187986 | 2334 | 1 | Updated RRM clause 6 tests to cover Cat-M2 requirement | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187987 | 2335 | 1 | Updated RRM clause 7 tests to cover Cat-M2 requirement | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187993 | 2321 | 1 | Update NB-IOT Random Access test cases 6.2.16, 6.2.17 and 6.2.18 | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187994 | 2319 | 1 | Update 8.16.78 to add TT information | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-188052 | 2351 | - | Editorial correction to FS3 RRM clause 8 Tests | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-188237 | 2329 | 1 | Correction to 8.26.7 FS3 Intra-frequency event triggered reporting | 15.3.0 | 15.4.0 |
| 2019-03 | RAN#83 | R5-191152 | 2352 | - | Introduction of RRM TC 9.1.68 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191161 | 2355 | - | Introduction of Rel-13 CA RRM TC 8.16.87 with TT | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191162 | 2356 | - | Introduction of Rel-13 CA RRM TC 8.16.88 with TT | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191163 | 2357 | - | Introduction of Rel-13 CA RRM TC 8.16.91 with TT | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191164 | 2358 | - | Introduction of Rel-13 CA RRM TC 8.16.92 with TT | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191165 | 2359 | - | Updated to Annex E and F for chapter 8 RRM Generic CA tests | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191502 | 2366 | - | Correction to test procedure for 8.16.39 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191574 | 2381 | - | Addition of new RRM feMTC Test Case 4.2.28 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191575 | 2382 | - | Addition of new RRM feMTC Test Case 4.2.29 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191580 | 2384 | - | Correction of RRM IncMon TC 4.2.10 including Test Tolerances | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191581 | 2385 | - | Correction of RRM IncMon TC 4.2.11 including Test Tolerances | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191582 | 2386 | - | Correction of RRM IncMon TC 4.3.1.5 including Test Tolerances | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191583 | 2387 | - | Correction of RRM IncMon TC 4.3.2A including Test Tolerances | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191584 | 2388 | - | Correction of RRM IncMon TC 4.3.3A including Test Tolerances | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191585 | 2389 | - | Correction of RRM IncMon TC 4.3.4.4 including Test Tolerances | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191586 | 2390 | - | Correction of RRM IncMon TC 8.4.8 including Test Tolerances | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191587 | 2391 | - | Correction of RRM IncMon TC 8.4.9 including Test Tolerances | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191588 | 2392 | - | Correction of RRM IncMon TC 8.5.8 including Test Tolerances | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191589 | 2393 | - | Correction of RRM IncMon TC 8.6.3 including Test Tolerances | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191590 | 2394 | - | Correction of RRM IncMon TC 8.7.5 including Test Tolerances | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191591 | 2395 | - | Correction of RRM IncMon TC 8.7A.1 including Test Tolerances | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191592 | 2396 | - | Correction of MU and TT in Annex F for IncMon RRM Test Cases | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191788 | 2400 | - | Correction to 8.26.5 and 8.26.7 E-UTRAN FDD-FS3 Intra-frequency event triggered reporting | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191789 | 2401 | - | Clean up of Cat-M1 event triggered reporting TC 8.1.26 and 8.1.27 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191790 | 2402 | - | Additional Message Exception for DRX Configuration for CatNB | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191964 | 2405 | - | Update to NB-IOT uplink timing test case. | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-191966 | 2407 | - | Update on Applicability of NB-IOT RACH test case with Non-anchor Carrier | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192191 | 2412 | - | Update on Applicability and Procedure for generic duplex mode RRM test case 8.16.85 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192192 | 2413 | - | Update on Applicability for generic duplex mode RRM test cases 8.16.83, 8.16.84 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192501 | 2353 | 1 | Introduction of RRM TC 9.2.55 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192502 | 2354 | 1 | Addition of TT Statement and Cell Mapping for TCs 9.1.68 & 9.2.55 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192525 | 2360 | 1 | Introduction of RRM TC 7.1.21 with TT | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192526 | 2361 | 1 | Introduction of RRM TC 7.1.22 with TT | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192527 | 2362 | 1 | Introduction of RRM TC 7.1.23 with TT | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192528 | 2363 | 1 | Introduction of RRM TC 7.1.24 with TT | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192529 | 2364 | 1 | Updated to Annex E and F for new feMTC RRM chap 7 tests | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192530 | 2383 | 1 | Addition of new RRM feMTC Test Case 4.2.30 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192537 | 2409 | 1 | Modifications to "E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN" Test Case | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192538 | 2410 | 1 | Modifications to "E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN" Test Case | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192627 | 2406 | 1 | Update to NB-IOT RLM cases | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192628 | 2403 | 1 | Update to 4 DL CA Activation and Deactivation with generic duplex modes | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192629 | 2404 | 1 | Update to 5 DL CA Activation and Deactivation with generic duplex modes | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192630 | 2408 | 1 | Update to generic duplex mode 3 DL CA deactivation test case | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192641 | 2367 | 1 | Band 53 introduction in 3GPP TS 36.521-3 | 15.5.0 | 16.0.0 |
| 2019-06 | RAN#84 | R5-193684 | 2414 | - | Introduction of LTE Band 85 in Group of band definitions | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-193881 | 2415 | - | Update of Annex F.3.2 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194184 | 2416 | - | Introduction of 3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194186 | 2417 | - | Introduction of 3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194219 | 2418 | - | Correction to UE Timing Advance Adjustment Accuracy Tests for CEModeA | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194220 | 2419 | - | Correction to Cat-M1 Radio Link Monitoring TC 7.3.52 and 7.3.53 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194221 | 2420 | - | Clean up 3DL and 4DL RSRP test with generic duplex modes | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194222 | 2421 | - | Clean up 3DL to 5DL CA Activation and Deactivation TCs | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194223 | 2422 | - | Clean up 3DL to 5DL CA RSRP and RSRQ tests | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194224 | 2423 | - | Editorial correction to Event Triggered Reporting on Deactivated SCell | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194225 | 2424 | - | Clean up 3DL to 5DL CA Event Triggered Reporting TCs | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194334 | 2425 | - | Introduction of 6DL CA RRM TC 8.16.95 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194335 | 2426 | - | Introduction of 7DL CA RRM TC 8.16.99 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194336 | 2427 | - | Insertion of test tolerance statements for new RRM test cases in Annex F | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194337 | 2428 | - | Correction of message content to RRM 5DL CA test cases | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194414 | 2430 | - | Correction random access tests for eMTC and NB-IoT | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194415 | 2431 | - | Spec alignment eMTC RMCs for 8.1.x | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194416 | 2432 | - | Correction 7.1.11 Test 1 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194417 | 2433 | - | systemInformationBlockType1Dedicated-r11 missing in eMTC HO tests | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195078 | 2434 | 1 | Correction to test case 8.16.76 of 36.521-3 | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195085 | 2429 | 1 | Completion 3DL CA generic tests | 16.0.0 | 16.1.0 |
| 2019-09 | RAN#85 | R5-195728 | 2438 | - | Update of 6.2.16 and 6.2.17 for alignment with core specification | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196452 | 2441 | - | Update to 3DL/3UL TDD CA for Non-Contention Based Random Access Test for 2 SCells | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196453 | 2442 | - | Update to 3DL/3UL TDD CA for UE Transmit Timing Accuracy Tests for 2 SCells | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196634 | 2444 | - | Band 73 information to E-UTRA band group in 36.521-3 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196686 | 2446 | - | Correction of RRM feMTC Test Case 4.2.28 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196687 | 2447 | - | Correction of RRM feMTC Test Case 4.2.29 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196688 | 2448 | - | Correction of RRM feMTC Test Case 4.2.30 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196689 | 2449 | - | Test system uncertainties and the Test Tolerances for RRM feMTC Test Cases | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196691 | 2450 | - | Addition of RRM feMTC Test Case 8.1.41 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196692 | 2451 | - | Addition of RRM feMTC Test Case 8.1.42 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196693 | 2452 | - | Addition of RRM feMTC Test Case 8.2.14 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197475 | 2439 | 1 | Correction 4CC and 5CC applicability - Chapter 8 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197476 | 2440 | 1 | Correction 4CC and 5CC applicability - Chapter 9 | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197660 | 2443 | 2 | Corrections to test case for HD FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage | 16.1.0 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197766 | 2457 | - | Cleaning up of R13 CA RRM test cases | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198021 | 2458 | - | Correction 6.1.15 | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198022 | 2459 | - | Correction feMTC applicability ch 4 | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198077 | 2460 | - | Update for removing editor’s note of V2V Test Cases | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198107 | 2461 | - | Update to 3DL/3UL TDD CA for UE Transmit Timing Accuracy and Non-Contention Based Random Access Tests for 2 SCells | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198516 | 2462 | - | Correction of PRACH-Configuration parameters for TC 5.1.13 | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198557 | 2463 | - | Updated to RRM 6DL and 7DL CA test cases | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198607 | 2464 | - | Cell configuration mapping for RRM feMTC Test Cases | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198608 | 2465 | - | Editorial correction of Annex A | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198609 | 2466 | - | Test Tolerances for RRM feMTC Test Cases in Annex F | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198612 | 2468 | - | Correction of RRM feMTC Test Case 8.1.41 | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198613 | 2469 | - | Correction of RRM feMTC Test Case 8.1.42 | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198614 | 2470 | - | Correction of RRM feMTC Test Case 8.2.14 | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198615 | 2471 | - | Editorial correction of RRM feMTC Test Cases | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198617 | 2473 | - | Addition of a new RRM feMTC Test Case 8.3.13 | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198621 | 2477 | - | Addition of a new RRM feMTC Test Case 8.3.17 | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198622 | 2478 | - | Addition of a new RRM feMTC Test Case 8.3.18 | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198623 | 2479 | - | Addition of a new RRM feMTC Test Case 8.3.19 | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198624 | 2480 | - | Addition of a new RRM feMTC Test Case 8.4.12 | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198625 | 2481 | - | Addition of a new RRM feMTC Test Case 8.4.13 | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198626 | 2482 | - | Addition of a new RRM feMTC Test Case 8.4.14 | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198627 | 2483 | - | Addition of a new RRM feMTC Test Case 8.4.15 | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198628 | 2484 | - | Editorial correction of RRM IncMon Test Cases | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199450 | 2472 | 1 | Addition of a new RRM feMTC Test Case 8.3.12 | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199451 | 2474 | 1 | Addition of a new RRM feMTC Test Case 8.3.14 | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199452 | 2475 | 1 | Addition of a new RRM feMTC Test Case 8.3.15 | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199453 | 2476 | 1 | Addition of a new RRM feMTC Test Case 8.3.16 | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199475 | 2453 | 1 | Addition of V2X RRM test cases 12.3.1 and 12.3.2 | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199476 | 2454 | 1 | Update of V2X RRM test case 12.4 | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199477 | 2455 | 1 | Addition of V2X RRM test case 12.5 | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199478 | 2456 | 1 | Addition of V2X RRM test cases 12.6.1 and 12.6.2 | 16.2.0 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200536 | 2486 | - | Correction of definitions of Bandwidth-reduced Low-complexity or Coverage Enhanced UE | 16.3.0 | 16.4.0 |
| 2020-03 | RAN#87 | R5-200537 | 2487 | - | Cell configuration mapping for RRM feMTC Test Cases | 16.3.0 | 16.4.0 |
| 2020-03 | RAN#87 | R5-200538 | 2488 | - | Addition of a new RRM feMTC Test Case 5.1.28 | 16.3.0 | 16.4.0 |
| 2020-03 | RAN#87 | R5-200539 | 2489 | - | Addition of a new RRM feMTC Test Case 5.1.34 | 16.3.0 | 16.4.0 |
| 2020-03 | RAN#87 | R5-200540 | 2490 | - | Addition of a new RRM feMTC Test Case 6.1.18 | 16.3.0 | 16.4.0 |
| 2020-03 | RAN#87 | R5-200541 | 2491 | - | Addition of a new RRM feMTC Test Case 7.3.69 | 16.3.0 | 16.4.0 |
| 2020-03 | RAN#87 | R5-200542 | 2492 | - | Correction of RRM feMTC Test Case 8.3.16 | 16.3.0 | 16.4.0 |
| 2020-03 | RAN#87 | R5-200543 | 2493 | - | Correction of RRM feMTC Test Case 8.3.17 | 16.3.0 | 16.4.0 |
| 2020-03 | RAN#87 | R5-200544 | 2494 | - | Correction of RRM feMTC Test Case 8.3.18 | 16.3.0 | 16.4.0 |
| 2020-03 | RAN#87 | R5-200545 | 2495 | - | Correction of RRM feMTC Test Case 8.3.19 | 16.3.0 | 16.4.0 |
| 2020-03 | RAN#87 | R5-200546 | 2496 | - | Correction of RRM feMTC Test Case 8.4.12 | 16.3.0 | 16.4.0 |
| 2020-03 | RAN#87 | R5-200547 | 2497 | - | Correction of RRM feMTC Test Case 8.4.13 | 16.3.0 | 16.4.0 |
| 2020-03 | RAN#87 | R5-200548 | 2498 | - | Correction of RRM feMTC Test Case 8.4.14 | 16.3.0 | 16.4.0 |
| 2020-03 | RAN#87 | R5-200549 | 2499 | - | Correction of RRM feMTC Test Case 8.4.15 | 16.3.0 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201608 | 2501 | - | Correction SIB5-NB 4.2.24 | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-201609 | 2502 | - | Correction RSRP-Threshold TC 6.2.16 | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202994 | 2503 | 1 | Align eMTC RLM test cases to core spec | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-201611 | 2504 | - | Correctly add evaluation limits 9.1.1.2_2 | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-201789 | 2505 | - | Updated to Annexes for Rel-15 NB-IoT test configuration | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202086 | 2506 | - | Editorial correction of clause 8.7A title | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202087 | 2507 | - | Test Tolerances for RRM feMTC Test Cases in Annex F | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202088 | 2508 | - | Correction of RRM feMTC Test Cases 8.3.12 to 8.3.19 with TT | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202787 | 2509 | 1 | Addition of 4.2.35 NB-IOT TDD-TDD Intra frequency cell reselection in normal coverage | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202788 | 2510 | 1 | Addition of 4.2.36 NB-IOT TDD-TDD Intra frequency cell reselection in enhanced coverage | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202789 | 2512 | 1 | Addition of 4.2.38 NB-IOT HD-FDD Intra frequency cell reselection in normal coverage with RRM measurement relaxation | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202790 | 2515 | 1 | Addition of 4.2.37 NB-IOT TDD-TDD Inter frequency cell reselection in enhanced coverage | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202799 | 2500 | 1 | Correction 4.2.18 PCID selection | 16.4.0 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202800 | 2513 | 1 | Correction of message contents to e-MTC Intra-frequency Handover | 16.4.0 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203800 | 2516 | - | Addition of 6.1.23 NB-IOT TDD Inter-frequency RRC Re-establishment in In-Band mode under normal coverage | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-203801 | 2517 | - | Addition of 6.1.24 NB-IOT TDD - TDD Intra-frequency RRC Re-establishment in In-Band mode under enhanced coverage | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-203802 | 2518 | - | Addition of 6.2.19 NB-IOT TDD Contention Based Random Access Test in normal coverage | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-203803 | 2519 | - | Addition of 6.2.20 NB-IOT TDD Contention Based Random Access Test in enhanced coverage | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-203933 | 2520 | - | Addition of sTTI related definitions, symbols and abbreviations to TS 36.521-3 | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-203936 | 2523 | - | Addition of downlink RMC configuration with sTTI | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-203997 | 2524 | - | Correction Test applicability of test case 9.2.56 and 9.2.57 | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204024 | 2525 | - | Correction of Table 9.2.18.1.3-2 and Table 9.2.18.2.3-2 | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204047 | 2527 | - | Correction to intra-frequency event triggered reporting test case in CEModeA | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204048 | 2528 | - | Correction to E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for CE UE in CEmodeA | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204109 | 2530 | - | Correction of RRM feMTC Test Case 5.1.34 including Test Tolerance | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204110 | 2531 | - | Correction of RRM feMTC Test Case 6.1.18 including Test Tolerance | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204111 | 2532 | - | Correction of RRM feMTC Test Case 7.3.69 including Test Tolerance | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204112 | 2533 | - | Correction of RRM feMTC Test Cases 8.4.12 - 15 including Test Tolerance | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204113 | 2534 | - | Test Tolerances for RRM feMTC Test Cases in Annex F | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204158 | 2535 | - | Correction to test applicability of NB-IoT RRM tests | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204731 | 2526 | 1 | Editorial update of Annex C.3.3.1 | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204798 | 2521 | 1 | Addition of new test case 7.2.13 E-UTRAN FDD Timing Advance Adjustment Delay Test for sTTI and short processing time | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204799 | 2522 | 1 | Addition of new test case 7.2.14 E-UTRAN TDD Timing Advance Adjustment Accuracy Test for sTTI and short processing time | 16.5.0 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204978 | 2529 | 1 | Correction of RRM feMTC Test Case 5.1.28 including Test Tolerance | 16.5.0 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205076 | 2539 |  | Void obsolete RRM test cases - Annex E | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205087 | 2540 |  | Introduction of LTE Bands 87 and 88 in Group of band definitions | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205693 | 2544 |  | Update to test applicability of NB-IoT RRM tests | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205768 | 2546 |  | Addition of conditions for sTTI and short processing time | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205849 | 2548 |  | Correction of PDSCH parameter for event triggered reporting TC 8.1.26 | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205850 | 2549 |  | Correction of sr-ConfigIndex for RSRP TC 9.1.53 | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205892 | 2551 |  | Update of 4.2.36 NB-IOT TDD-TDD Intra frequency cell reselection in enhanced coverage | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205893 | 2552 |  | Update of 4.2.37 NB-IOT TDD-TDD Inter frequency cell reselection in enhanced coverage | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205895 | 2553 |  | Update of 4.2.38 NB-IOT TDD-TDD Intra frequency cell reselection in normal coverage with WUS | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205897 | 2554 |  | Update of Annex E and F for NB-IOT TDD RRM test cases 4.2.35 to 4.2.38 | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-206710 | 2545 | 1 | Update of UE Timing Advance Adjustment Accuracy Test for sTTI | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-206755 | 2547 | 1 | Update of NB-IoT Test Cases in TDD mode | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-206756 | 2550 | 1 | Update of 4.2.35 NB-IOT TDD-TDD Intra frequency cell reselection in normal coverage | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-206780 | 2536 | 1 | Void obsolete RRM 3CC test cases - Chapter 8 | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-206781 | 2537 | 1 | Void obsolete RRM CA test cases - Chapter 9 | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-206782 | 2538 | 1 | Void obsolete RRM 4CC and 5CC test cases - Chapter 8 | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-206898 | 2541 | 1 | Adding new TC 9.1.71 6 DL RSRP for E-UTRAN in CA | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-206899 | 2542 | 1 | Adding new TC 9.2.58 6 DL RSRQ for E-UTRAN in CA | 16.6.0 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210890 | 2567 | - | Addition of Cell configuration mapping for NB-IoT RRM TDD Test Cases in Annex E | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-210892 | 2569 | - | Update of MU for addition of new NB-IoT TDD Test Cases | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-211753 | 2555 | 1 | Addition of new test case 9.1.72 | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-211754 | 2556 | 1 | Addition of new test case 9.2.59 | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-211758 | 2573 | 1 | Introduction of MU and TT for CA Idle Mode Measurements | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-211759 | 2570 | 1 | FDD Inter Frequency Absolute RSRP Accuracy CA Idle Mode Measurements for Non-Overlapping Carrier | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-211760 | 2571 | 1 | FDD Inter Frequency Absolute RSRP Accuracy CA Idle Mode Measurements for Overlapping Carrier | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-211761 | 2572 | 1 | FDD Intra Frequency Absolute RSRP Accuracy CA Idle Mode Measurements | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-211790 | 2561 | 1 | Addition of 6.2.21 TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-211791 | 2562 | 1 | Addition of 7.1.27 E-UTRAN TDD UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-211792 | 2563 | 1 | Addition of 7.1.28 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-211793 | 2564 | 1 | Addition of 7.2.15 E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-211794 | 2565 | 1 | Addition of 7.3.88 TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-211795 | 2566 | 1 | Addition of 7.3.89 TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-211847 | 2568 | 1 | Correction of Table number for RRM Radio Link Monitoring Test Cases | 16.7.0 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212879 | 2580 | - | Addition of Cell configuration mapping for NB-IoT RRM TDD Test Cases in Annex E | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212880 | 2581 | - | Update of MU for addition of new NB-IoT TDD Test Cases | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-213126 | 2582 | - | Cell Configuration Mapping for Absolute RSRP and RSRQ Accuracy for CA Idle Mode Measurements | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-213128 | 2583 | - | Introduction of MU and TT for RSRQ CA Idle Mode Measurements | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-213130 | 2584 | - | FDD Intra Frequency Absolute RSRQ Accuracy CA Idle Mode Measurements | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-213159 | 2585 | - | FDD Inter Frequency Absolute RSRQ Accuracy CA Idle Mode Measurements for Non-Overlapping Carrier | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-213162 | 2586 | - | FDD Inter Frequency Absolute RSRQ Accuracy CA Idle Mode Measurements for Overlapping Carrier | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-213985 | 2574 | 1 | Addition of 7.3.90 TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-213986 | 2575 | 1 | Addition of 7.3.91 TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-213987 | 2576 | 1 | Addition of 7.3.92 TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-213988 | 2577 | 1 | Addition of 7.3.93 TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-213989 | 2578 | 1 | Addition of 7.3.94 TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-213990 | 2579 | 1 | Addition of 7.3.95 TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage | 16.8.0 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214925 | 2587 | - | Correction of eMTC PRACH tests | 16.9.0 | 16.10.0 |
| 2021-09 | RAN#93 | R5-215001 | 2588 | - | Addition of BG_offset definition for LTE band groups | 16.9.0 | 16.10.0 |
| 2021-09 | RAN#93 | R5-215552 | 2591 | - | New testcase for E-UTRAN FDD hibernation and activation of known SCell in Non-DRX | 16.9.0 | 16.10.0 |
| 2021-09 | RAN#93 | R5-215553 | 2592 | - | New testcase for E-UTRAN FDD hibernation and activation of unknown SCell in Non-DRX | 16.9.0 | 16.10.0 |
| 2021-09 | RAN#93 | R5-215554 | 2593 | - | New testcase for E-UTRAN TDD hibernation and activation of known SCell in Non-DRX | 16.9.0 | 16.10.0 |
| 2021-09 | RAN#93 | R5-215561 | 2594 | - | New testcase for E-UTRAN TDD hibernation and activation of unknown SCell in Non-DRX | 16.9.0 | 16.10.0 |
| 2021-09 | RAN#93 | R5-215924 | 2590 | 1 | Update applicability of TC 9.2.58 | 16.9.0 | 16.10.0 |
| 2021-09 | RAN#93 | R5-216140 | 2589 | 1 | Removing editors note from NB TDD RLM test cases | 16.9.0 | 16.10.0 |
| 2021-12 | RAN#94 | R5-216836 | 2595 | - | Update applicability of TC 9.1.71 | 16.10.0 | 16.11.0 |
| 2021-12 | RAN#94 | R5-216837 | 2596 | - | Update applicability of TC 9.1.72 | 16.10.0 | 16.11.0 |
| 2021-12 | RAN#94 | R5-216838 | 2597 | - | Update applicability of TC 9.2.59 | 16.10.0 | 16.11.0 |
| 2022-03 | RAN#95 | R5-220824 | 2612 | - | Addition of 5.1.41 inter-frequency DAPS handover | 16.11.0 | 16.12.0 |
| 2022-03 | RAN#95 | R5-221305 | 2613 | - | Addition of Intra frequency conditional handover Test Case 5.1.47 | 16.11.0 | 16.12.0 |
| 2022-03 | RAN#95 | R5-221306 | 2614 | - | Addition of Intra frequency conditional handover Test Case 5.1.48 | 16.11.0 | 16.12.0 |
| 2022-03 | RAN#95 | R5-221307 | 2615 | - | Addition of Inter frequency conditional handover Test Case 5.1.49 | 16.11.0 | 16.12.0 |
| 2022-03 | RAN#95 | R5-221308 | 2616 | - | Addition of Inter frequency conditional handover Test Case 5.1.50 | 16.11.0 | 16.12.0 |
| 2022-03 | RAN#95 | R5-221309 | 2617 | - | Addition of Inter frequency conditional handover Test Case 5.1.51 | 16.11.0 | 16.12.0 |
| 2022-03 | RAN#95 | R5-221310 | 2618 | - | Addition of Inter frequency conditional handover Test Case 5.1.52 | 16.11.0 | 16.12.0 |
| 2022-03 | RAN#95 | R5-221311 | 2619 | - | Addition of cell configuration mapping for conditional handover test cases in Annex E | 16.11.0 | 16.12.0 |
| 2022-03 | RAN#95 | R5-221755 | 2610 | 1 | Removing editors note in CA test cases | 16.11.0 | 16.12.0 |
| 2022-03 | RAN#95 | R5-221756 | 2611 | 1 | Addition of new test case in Annex F.1.2 and Annex F.3.2 | 16.11.0 | 16.12.0 |
| 2022-03 | RAN#95 | R5-221801 | 2598 | 1 | Addition of new feMob test case 5.1.42 | 16.11.0 | 16.12.0 |
| 2022-03 | RAN#95 | R5-221802 | 2599 | 1 | Addition of new feMob test case 5.1.43 | 16.11.0 | 16.12.0 |
| 2022-03 | RAN#95 | R5-221803 | 2600 | 1 | Addition of new feMob test case 5.1.44 | 16.11.0 | 16.12.0 |
| 2022-03 | RAN#95 | R5-221804 | 2601 | 1 | Addition of new feMob test case 5.1.45 | 16.11.0 | 16.12.0 |
| 2022-03 | RAN#95 | R5-221805 | 2602 | 1 | Addition of new feMob test case 5.1.46 | 16.11.0 | 16.12.0 |
| 2022-03 | RAN#95 | R5-221806 | 2603 | 1 | Addition of new feMob test case 5.1.53 | 16.11.0 | 16.12.0 |
| 2022-03 | RAN#95 | R5-221807 | 2604 | 1 | Addition of new feMob test case 5.1.54 | 16.11.0 | 16.12.0 |
| 2022-03 | RAN#95 | R5-221808 | 2605 | 1 | Addition of new feMob test case 5.1.55 | 16.11.0 | 16.12.0 |
| 2022-03 | RAN#95 | R5-221809 | 2606 | 1 | Addition of new feMob test case 5.1.56 | 16.11.0 | 16.12.0 |
| 2022-03 | RAN#95 | R5-221810 | 2607 | 1 | Addition of new feMob test case 5.1.57 | 16.11.0 | 16.12.0 |
| 2022-03 | RAN#95 | R5-221811 | 2608 | 1 | Addition of new feMob test case 5.1.58 | 16.11.0 | 16.12.0 |
| 2022-06 | RAN#96 | R5-222976 | 2621 | - | Update of feMob test case 5.1.42 | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-222977 | 2622 | - | Update of feMob test case 5.1.43 | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-222978 | 2623 | - | Update of feMob test case 5.1.44 | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-222979 | 2624 | - | Update of feMob test case 5.1.45 | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-222980 | 2625 | - | Update of feMob test case 5.1.46 | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-222981 | 2626 | - | Update of feMob test case 5.1.53 | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-222982 | 2627 | - | Update of feMob test case 5.1.54 | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-222983 | 2628 | - | Update of feMob test case 5.1.55 | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-222984 | 2629 | - | Update of feMob test case 5.1.56 | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-222985 | 2630 | - | Update of feMob test case 5.1.57 | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-222986 | 2631 | - | Update of feMob test case 5.1.58 | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-223177 | 2632 | - | Correction of Intra frequency conditional handover Test Case 5.1.47 including Test Tolerance | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-223178 | 2633 | - | Correction of Intra frequency conditional handover Test Case 5.1.48 including Test Tolerance | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-223179 | 2634 | - | Correction of Inter frequency conditional handover Test Case 5.1.49 including Test Tolerance | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-223181 | 2636 | - | Correction of Inter frequency conditional handover Test Case 5.1.51 including Test Tolerance | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-223182 | 2637 | - | Correction of Inter frequency conditional handover Test Case 5.1.52 including Test Tolerance | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-223183 | 2638 | - | Addition of LTE RRM CHO test cases Test Tolerance into Annex F | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-223188 | 2640 | - | Addition of 6 DL CA Activation and Deactivation of Known SCell Test Case 8.16.97 | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-223189 | 2641 | - | Addition of 6 DL CA Activation and Deactivation of Unknown SCell Test Case 8.16.98 | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-223190 | 2642 | - | Addition of 7 DL CA Event Triggered Reporting on Deactivated SCell Test Case 8.16.100 | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-223191 | 2643 | - | Addition of 7 DL CA Activation and Deactivation of Known SCell Test Case 8.16.101 | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-223192 | 2644 | - | Addition of 7 DL CA Activation and Deactivation of Unknown SCell Test Case 8.16.102 | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-223193 | 2645 | - | Correction of minimum conformance requirements for RRM 3CC, 4CC and 5CC test cases | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-223294 | 2646 | - | Update Annex E and F for feMob test cases | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-223648 | 2639 | 1 | Addition of 6 DL CA Event Triggered Reporting on Deactivated SCell test case 8.16.96 | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-223702 | 2635 | 1 | Correction of Inter frequency conditional handover Test Case 5.1.50 including Test Tolerance | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-223794 | 2620 | 1 | Alignment of euCA RRM testcase numbering according to 36.133 | 16.12.0 | 16.13.0 |
| 2022-09 | RAN#97 | R5-224840 | 2652 | - | Update to V2X test case 12.1.2 | 16.13.0 | 16.14.0 |
| 2022-09 | RAN#97 | R5-225156 | 2653 | - | Editorial Correction of 5 DL CA Event Triggered Reporting on Deactivated SCell test case 8.16.92 | 16.13.0 | 16.14.0 |
| 2022-09 | RAN#97 | R5-225157 | 2654 | - | Correction of 6 DL CA Event Triggered Reporting on Deactivated SCell test case 8.16.96 | 16.13.0 | 16.14.0 |
| 2022-09 | RAN#97 | R5-225158 | 2655 | - | Correction of 6 DL CA Activation and Deactivation of Known SCell Test Case 8.16.97 | 16.13.0 | 16.14.0 |
| 2022-09 | RAN#97 | R5-225159 | 2656 | - | Correction of 6 DL CA Activation and Deactivation of Unknown SCell Test Case 8.16.98 | 16.13.0 | 16.14.0 |
| 2022-09 | RAN#97 | R5-225160 | 2657 | - | Correction of 7 DL CA Event Triggered Reporting on Deactivated SCell Test Case 8.16.100 | 16.13.0 | 16.14.0 |
| 2022-09 | RAN#97 | R5-225161 | 2658 | - | Correction of 7 DL CA Activation and Deactivation of Known SCell Test Case 8.16.101 | 16.13.0 | 16.14.0 |
| 2022-09 | RAN#97 | R5-225162 | 2659 | - | Correction of 7 DL CA Activation and Deactivation of Unknown SCell Test Case 8.16.102 | 16.13.0 | 16.14.0 |
| 2022-09 | RAN#97 | R5-225163 | 2660 | - | Correction of Cell Configuration in Annex E for 6 and 7 DL CA test cases | 16.13.0 | 16.14.0 |
| 2022-09 | RAN#97 | R5-225164 | 2661 | - | Correction of MTSU and TT in Annex F for 6 and 7 DL CA test cases | 16.13.0 | 16.14.0 |
| 2022-09 | RAN#97 | R5-225837 | 2647 | 1 | Correction of Default Configuration Parameters for Test 1 in Test Case 7.1.11 | 16.13.0 | 16.14.0 |
| 2022-09 | RAN#97 | R5-224828 | 2651 | - | Introduction of LTE Band 103 in Group of band definitions | 16.14.0 | 17.0.0 |
| 2022-12 | RAN#98 | R5-226277 | 2663 | - | Correction of table numbers in 7.1.4.3 | 17.0.0 | 17.1.0 |
| 2022-12 | RAN#98 | R5-226278 | 2664 | - | Correction of table number, clause numbers and table tile in 7.1.4_1 | 17.0.0 | 17.1.0 |
| 2022-12 | RAN#98 | R5-226477 | 2665 | - | Inclusive Language Review of LTE RRM Section 4 | 17.0.0 | 17.1.0 |
| 2022-12 | RAN#98 | R5-226478 | 2666 | - | Inclusive Language Review of Event reporting LTE RRM test cases | 17.0.0 | 17.1.0 |
| 2022-12 | RAN#98 | R5-226479 | 2667 | - | Inclusive Language Review of CA Event reporting LTE RRM test cases | 17.0.0 | 17.1.0 |
| 2022-12 | RAN#98 | R5-226709 | 2668 | - | Inclusive Language Review of LTE RRM Annex | 17.0.0 | 17.1.0 |
| 2023-03 | RAN#99 | R5-230399 | 2669 | - | Update to abbreviations of E-UTRA RRM TCs for IoT-NTN | 17.1.0 | 18.0.0 |  |
| 2023-03 | RAN#99 | R5-230400 | 2670 | - | Addition of groups of bands for satellite access TC 3.5.1A | 17.1.0 | 18.0.0 |  |
| 2023-06 | RAN#100 | R5-232324 | 2671 | - | Updates to groups of band as part of introduction of LTE Band 54 | 18.0.0 | 18.1.0 |  |
| 2023-09 | RAN#101 | R5-235183 | 2700 | - | New Annex I.8 definition for High level test procedure for SAN RRM tests | 18.1.0 | 18.2.0 |  |
| 2023-09 | RAN#101 | R5-235758 | 2672 | 1 | Addition of new test case 13.4.3.1 RLM for OOS with DRX in normal coverage | 18.1.0 | 18.2.0 |  |
| 2023-09 | RAN#101 | R5-235759 | 2680 | 1 | Addition of new test case 13.4.2.1 for TA adjustment accuracy test in normal coverage | 18.1.0 | 18.2.0 |  |
| 2023-09 | RAN#101 | R5-235760 | 2685 | 1 | Addition of cell configuration mapping for IOT NTN cases into Annex E | 18.1.0 | 18.2.0 |  |
| 2023-09 | RAN#101 | R5-235763 | 2673 | 1 | Addition of new test case 13.4.3.2 RLM for OOS with DRX in enhanced coverage | 18.1.0 | 18.2.0 |  |
| 2023-09 | RAN#101 | R5-235764 | 2674 | 1 | Addition of new test case 13.4.3.3 RLM for IS with DRX in enhanced coverage | 18.1.0 | 18.2.0 |  |
| 2023-09 | RAN#101 | R5-235765 | 2675 | 1 | Addition of new test case 13.4.3.4 RLM for IS with DRX in normal coverage | 18.1.0 | 18.2.0 |  |
| 2023-09 | RAN#101 | R5-235766 | 2676 | 1 | Addition of new test case 13.4.3.5 RLM for IS without DRX in normal coverage | 18.1.0 | 18.2.0 |  |
| 2023-09 | RAN#101 | R5-235767 | 2677 | 1 | Addition of new test case 13.4.3.6 RLM for IS without DRX in enhanced coverage | 18.1.0 | 18.2.0 |  |
| 2023-09 | RAN#101 | R5-235768 | 2678 | 1 | Addition of new test case 13.4.3.7 RLM for OOS without DRX in normal coverage | 18.1.0 | 18.2.0 |  |
| 2023-09 | RAN#101 | R5-235769 | 2679 | 1 | Addition of new test case 13.4.3.8 RLM for OOS without DRX in enhanced coverage | 18.1.0 | 18.2.0 |  |
| 2023-09 | RAN#101 | R5-235770 | 2681 | 1 | Addition of new test case 13.4.2.2 for TA adjustment accuracy test in enhanced coverage | 18.1.0 | 18.2.0 |  |
| 2023-09 | RAN#101 | R5-235771 | 2682 | 1 | Addition of new test case 13.4.1.1 for Transmit Timing Accuracy test in normal coverage | 18.1.0 | 18.2.0 |  |
| 2023-09 | RAN#101 | R5-235772 | 2683 | 1 | Addition of new test case 13.4.1.2 for Transmit Timing Accuracy test in enhanced coverage | 18.1.0 | 18.2.0 |  |
| 2023-09 | RAN#101 | R5-235773 | 2684 | 1 | Addition of new test case 13.4.1.3 for Transmit Timing Accuracy test under enhanced coverage with segment transmission in NGSO. | 18.1.0 | 18.2.0 |  |
| 2023-09 | RAN#101 | R5-235783 | 2689 | 1 | Addition of new test case 13.3.2.1 for NTN Random Access in normal coverage | 18.1.0 | 18.2.0 |  |
| 2023-09 | RAN#101 | R5-235784 | 2698 | 1 | Addition of new test case 13.3.1.1 ReEST under normal coverage | 18.1.0 | 18.2.0 |  |
| 2023-09 | RAN#101 | R5-235785 | 2699 | 1 | Addition of new test case 13.3.1.2 ReEST under enhanced coverage | 18.1.0 | 18.2.0 |  |
| 2023-09 | RAN#101 | R5-235787 | 2690 | 1 | Addition of new test case 13.3.2.2 for NTN Random Access in enhanced coverage | 18.1.0 | 18.2.0 |  |
| 2023-09 | RAN#101 | R5-235788 | 2691 | 1 | Addition of new test case 13.3.2.3 for NTN Random Access on Non-anchor Carrier in enhanced coverage | 18.1.0 | 18.2.0 |  |
| 2023-09 | RAN#101 | R5-235789 | 2692 | 1 | Addition of new test case 13.6.2.1 for NTN Downlink channel quality reporting accuracy under normal coverage | 18.1.0 | 18.2.0 |  |
| 2023-09 | RAN#101 | R5-235790 | 2693 | 1 | Addition of new test case 13.6.2.2 for NTN Downlink channel quality reporting accuracy under enhanced coverage | 18.1.0 | 18.2.0 |  |
| 2023-09 | RAN#101 | R5-235791 | 2694 | 1 | Addition of new test case 13.6.2.3 for NTN Downlink channel quality reporting accuracy on non-anchor carrier under normal coverage | 18.1.0 | 18.2.0 |  |
| 2023-09 | RAN#101 | R5-235792 | 2695 | 1 | Addition of new test case 13.6.2.4 for NTN Downlink channel quality reporting accuracy on non-anchor carrier under enhanced coverage | 18.1.0 | 18.2.0 |  |
| 2023-09 | RAN#101 | R5-235793 | 2696 | 1 | Addition of new test case 13.6.2.5 for NTN Downlink channel quality reporting accuracy in RRC_CONNECTED under normal coverage | 18.1.0 | 18.2.0 |  |
| 2023-09 | RAN#101 | R5-235794 | 2697 | 1 | Addition of new test case 13.6.2.6 for NTN Downlink channel quality reporting accuracy in RRC_CONNECTED under enhanced coverage | 18.1.0 | 18.2.0 |  |
| 2023-09 | RAN#101 | R5-235927 | 2686 | 1 | Addition of new test case 13.1.1.1 for cell reselection in normal coverage | 18.1.0 | 18.2.0 |  |
| 2023-09 | RAN#101 | R5-235928 | 2687 | 1 | Addition of new test case 13.1.1.2 for cell reselection in normal coverage with serving cell RRM measurement relaxation | 18.1.0 | 18.2.0 |  |
| 2023-09 | RAN#101 | R5-235929 | 2688 | 1 | Addition of new test case 13.1.1.3 for cell reselection in normal coverage with UE specific DRX | 18.1.0 | 18.2.0 |  |
| 2023-12 | RAN#102 | R5-235997 | 2701 | - | Update NB-IoT NTN RLM IS test cases including TTs | 18.2.0 | 18.3.0 |  |
| 2023-12 | RAN#102 | R5-235999 | 2702 | - | Update NB-IoT NTN RLM OOS test cases including TTs | 18.2.0 | 18.3.0 |  |
| 2023-12 | RAN#102 | R5-236001 | 2703 | - | Update NB-IoT NTN timing advance test cases including TTs | 18.2.0 | 18.3.0 |  |
| 2023-12 | RAN#102 | R5-236003 | 2704 | - | Update NB-IoT NTN timing accuracy test cases including TTs | 18.2.0 | 18.3.0 |  |
| 2023-12 | RAN#102 | R5-236004 | 2705 | - | Correction of NB-IoT NTN Cell reselection test cases 13.1.1.1 and 13.1.1.3 with Test Tolerance | 18.2.0 | 18.3.0 |  |
| 2023-12 | RAN#102 | R5-236005 | 2706 | - | Correction of NB-IoT NTN Cell reselection test cases 13.1.1.2 with Test Tolerance | 18.2.0 | 18.3.0 |  |
| 2023-12 | RAN#102 | R5-236007 | 2707 | - | Correction of NB-IoT NTN RRC Re-establishment test case 13.3.1.1 with Test Tolerance | 18.2.0 | 18.3.0 |  |
| 2023-12 | RAN#102 | R5-236008 | 2708 | - | Correction of NB-IoT NTN RRC Re-establishment test case 13.3.1.2 with Test Tolerance | 18.2.0 | 18.3.0 |  |
| 2023-12 | RAN#102 | R5-236028 | 2710 | - | Update of Annex F Measurement Uncertainties and Test Tolerances for IoT NTN random access and downlink channel quality reporting accuracy test cases | 18.2.0 | 18.3.0 |  |
| 2023-12 | RAN#102 | R5-236045 | 2712 | - | Several modification and correction of IoT NTN DCQR test cases | 18.2.0 | 18.3.0 |  |
| 2023-12 | RAN#102 | R5-236047 | 2713 | - | Test applicability update for several iot ntn RRM cases | 18.2.0 | 18.3.0 |  |
| 2023-12 | RAN#102 | R5-236206 | 2715 | - | Update of core requirement for NB-IOT NTN timing test cases | 18.2.0 | 18.3.0 |  |
| 2023-12 | RAN#102 | R5-236465 | 2718 | - | Addition of references of 36.521-3 | 18.2.0 | 18.3.0 |  |
| 2023-12 | RAN#102 | R5-236614 | 2720 | - | Correction to test environment for IoT NTN RA test cases | 18.2.0 | 18.3.0 |  |
| 2023-12 | RAN#102 | R5-236758 | 2722 | - | Core spec alignment for NB-IoT NTN cell reselection test cases | 18.2.0 | 18.3.0 |  |
| 2023-12 | RAN#102 | R5-237671 | 2709 | 1 | Deletion of editors notes of MU TT and update TT value for IoT NTN RA test cases | 18.2.0 | 18.3.0 |  |
| 2023-12 | RAN#102 | R5-237672 | 2721 | 1 | Deletion of editors notes of MU TT and update TT value for IoT NTN DCQR test cases | 18.2.0 | 18.3.0 |  |
| 2023-12 | RAN#102 | R5-237826 | 2719 | 1 | Update to test applicability of test case 8.2.7 | 18.2.0 | 18.3.0 |  |
| 2023-12 | RAN#102 | R5-237867 | 2714 | 1 | Update initial condition for NB-IOT NTN test cases. | 18.2.0 | 18.3.0 |  |
| 2023-12 | RAN#102 | R5-237868 | 2716 | 1 | Update of satellite configuration, npush-TXDuraion and power level for NB-IOT NTN timing test cases | 18.2.0 | 18.3.0 |  |
| 2023-12 | RAN#102 | R5-237869 | 2717 | 1 | Addition of serving satellite config for NB-IOT NTN test cases into Annex | 18.2.0 | 18.3.0 |  |
| 2023-12 | RAN#102 | R5-237937 | 2711 | 1 | Deletion of brackets editors notes and core spec alignment update for IoT NTN RA test cases | 18.2.0 | 18.3.0 |  |
| 2023-12 | RAN#102 | R5-237958 | 2724 | 1 | Updates to High level test procedure for SAN RRM tests | 18.2.0 | 18.3.0 |  |
| 2023-12 | RAN#102 | R5-237964 | 2723 | 1 | Corrections to UE position provision for IoT-NTN | 18.2.0 | 18.3.0 |  |
| 2024-03 | RAN#103 | R5-240024 | 2998 | - | Update NB-IoT NTN UL timing accuracy cases | 18.3.0 | 18.4.0 |  |
| 2024-03 | RAN#103 | R5-240029 | 2999 | - | Update NB-IoT NTN UL timing accuracy cases TT in Annex | 18.3.0 | 18.4.0 |  |
| 2024-03 | RAN#103 | R5-240211 | 3005 | - | Update TT value to message content and test requirement of RA cases | 18.3.0 | 18.4.0 |  |
| 2024-03 | RAN#103 | R5-240219 | 3008 | - | Update cell specific parameters for NB-IoT NTN RA test cases | 18.3.0 | 18.4.0 |  |
| 2024-03 | RAN#103 | R5-240615 | 3009 | - | NTN NB-IoT - Message exception correction in RRM test case 13.1.1.2 | 18.3.0 | 18.4.0 |  |
| 2024-03 | RAN#103 | R5-240922 | 3010 | - | Editorial update for UE timing advance for satellite access TC 13.4.2.2 | 18.3.0 | 18.4.0 |  |
| 2024-03 | RAN#103 | R5-241244 | 3012 | - | Core Spec alignment for s-IntraSearchP IE for Re-selection NB-IOT NTN RRM TCs | 18.3.0 | 18.4.0 |  |
| 2024-03 | RAN#103 | R5-241245 | 3013 | - | Re-Addition of omitted change for RRM test case 13.1.1.2 | 18.3.0 | 18.4.0 |  |
| 2024-03 | RAN#103 | R5-241246 | 3014 | - | Editorial corrections to NB-IOT NTN RRM Reselection test cases | 18.3.0 | 18.4.0 |  |
| 2024-03 | RAN#103 | R5-241248 | 3016 | - | Core Spec alignment for RRM NB-IOT NTN test cases | 18.3.0 | 18.4.0 |  |
| 2024-03 | RAN#103 | R5-241250 | 3018 | - | Core Spec alignment for RRM NB-IOT NTN UL Timing test cases | 18.3.0 | 18.4.0 |  |
| 2024-03 | RAN#103 | R5-241385 | 3019 | - | Channel quality reporting updates | 18.3.0 | 18.4.0 |  |
| 2024-03 | RAN#103 | R5-241386 | 3020 | - | Radio Link Monitoring updates | 18.3.0 | 18.4.0 |  |
| 2024-03 | RAN#103 | R5-241676 | 3021 | 1 | Annex A.3 update | 18.3.0 | 18.4.0 |  |
| 2024-03 | RAN#103 | R5-241794 | 2997 | 1 | Editorial corrections for NB-IoT NTN TA cases and RLM cases | 18.3.0 | 18.4.0 |  |
| 2024-03 | RAN#103 | R5-241795 | 3006 | 1 | Editorial removal to editors notes of DCQR cases | 18.3.0 | 18.4.0 |  |
| 2024-03 | RAN#103 | R5-241796 | 3015 | 1 | Editorial corrections to NB-IOT NTN RRM TCs | 18.3.0 | 18.4.0 |  |
| 2024-03 | RAN#103 | R5-242003 | 3011 | 1 | Correction to UE timing advance for satellite access TC 13.4.2.1 | 18.3.0 | 18.4.0 |  |
| 2024-03 | RAN#103 | R5-242004 | 3003 | 1 | Addition and correction to the NTN related abbreviations in 36.521-3 | 18.3.0 | 18.4.0 |  |
| 2024-06 | RAN#104 | R5-242120 | 3022 | - | Editorial correction to TC 6.2.16 | 18.4.0 | 18.5.0 |  |
| 2024-06 | RAN#104 | R5-242390 | 3024 | - | Addition of band groups for band 106 | 18.4.0 | 18.5.0 |  |
| 2024-06 | RAN#104 | R5-242416 | 3025 | - | Editorial correction to the wrong citation of rel-15 RRM cases | 18.4.0 | 18.5.0 |  |
| 2024-06 | RAN#104 | R5-242417 | 3026 | - | Editorial correction to the wrong message content of NTN RA cases | 18.4.0 | 18.5.0 |  |
| 2024-06 | RAN#104 | R5-242419 | 3027 | - | Addition of Test configuration for satellite access | 18.4.0 | 18.5.0 |  |
| 2024-06 | RAN#104 | R5-242420 | 3028 | - | Addition of Antenna configuration for satellite access | 18.4.0 | 18.5.0 |  |
| 2024-06 | RAN#104 | R5-242424 | 3030 | - | Update TT for NB-IoT NTN timing accuracy test cases | 18.4.0 | 18.5.0 |  |
| 2024-06 | RAN#104 | R5-242909 | 3035 | - | Correction to L2U redirection RRM test case 6.3.12 | 18.4.0 | 18.5.0 |  |
| 2024-06 | RAN#104 | R5-243053 | 3037 | - | Editorial corrections for IoT NTN RRM TCs | 18.4.0 | 18.5.0 |  |
| 2024-06 | RAN#104 | R5-243060 | 3041 | - | Addition of cell configuration mapping for IoT NTN RRM TCs | 18.4.0 | 18.5.0 |  |
| 2024-06 | RAN#104 | R5-243687 | 3042 | 1 | Update applicability for NB-NTN Channel quality reporting accuracy testcases | 18.4.0 | 18.5.0 |  |
| 2024-06 | RAN#104 | R5-243688 | 3038 | 1 | Addition of new RRM TC 13.5.1 for IoT NTN enh | 18.4.0 | 18.5.0 |  |
| 2024-06 | RAN#104 | R5-243689 | 3039 | 1 | Addition of new RRM TC 13.5.2 for IoT NTN enh | 18.4.0 | 18.5.0 |  |
| 2024-06 | RAN#104 | R5-243690 | 3040 | 1 | Addition of new RRM TC 13.5.3 for IoT NTN enh | 18.4.0 | 18.5.0 |  |
| 2024-06 | RAN#104 | R5-243809 | 3036 | 1 | Correction to LTE V2V and V2X RRM test cases | 18.4.0 | 18.5.0 |  |
| 2024-06 | RAN#104 | R5-243887 | 3034 | 1 | Update to NTN DCQR cases | 18.4.0 | 18.5.0 |  |
| 2024-06 | RAN#104 | R5-244018 | 3029 | 2 | Update NB-IoT NTN timing advance test cases | 18.4.0 | 18.5.0 |  |
| 2024-06 | RAN#104 | R5-244019 | 3031 | 1 | Update NB-IoT NTN test case TC 13.4.1.1 | 18.4.0 | 18.5.0 |  |
| 2024-09 | RAN#105 | R5-244219 | 3044 | - | Addition of IoT NTN band 254 in band group list | 18.5.0 | 18.6.0 |  |
| 2024-09 | RAN#105 | R5-244340 | 3046 | - | Correction to setup for SIB31 and SIB31-NB | 18.5.0 | 18.6.0 |  |
| 2024-09 | RAN#105 | R5-244341 | 3047 | - | Editorial update of test procedure in NTN DCQR cases | 18.5.0 | 18.6.0 |  |
| 2024-09 | RAN#105 | R5-245032 | 3052 | - | Update_to_RSRP-ThresholdsNPRACH-InfoList for NB-IoT RACH cases | 18.5.0 | 18.6.0 |  |
| 2024-09 | RAN#105 | R5-245033 | 3053 | - | Update_to_RSRP-ThresholdsNPRACH-InfoList for NTN RACH cases | 18.5.0 | 18.6.0 |  |
| 2024-09 | RAN#105 | R5-246018 | 3043 | 1 | Correction to IoT NTN RRM - 13.4.3.x test case | 18.5.0 | 18.6.0 |  |
| 2024-12 | RAN#106 | R5-246284 | 3054 | - | Addition of new test case 13.1.1.1_1 for NB IoT NTN | 18.6.0 | 18.7.0 |  |
| 2024-12 | RAN#106 | R5-246287 | 3057 | - | Addition of new test case 13.1.1.2_1 for NB IoT NTN | 18.6.0 | 18.7.0 |  |
| 2024-12 | RAN#106 | R5-246288 | 3058 | - | Addition of new test case 13.1.1.3_1 for NB IoT NTN | 18.6.0 | 18.7.0 |  |
| 2024-12 | RAN#106 | R5-246313 | 3060 | - | Update to message content of RSRP Range for NTN RACH cases | 18.6.0 | 18.7.0 |  |
| 2024-12 | RAN#106 | R5-246348 | 3064 | - | Addition of IoT NTN band 253 in band group list | 18.6.0 | 18.7.0 |  |
| 2024-12 | RAN#106 | R5-246392 | 3065 | - | Addition of maximum NPDCCH repetition number for NB-IoT NTN RLM IS test cases | 18.6.0 | 18.7.0 |  |
| 2024-12 | RAN#106 | R5-247286 | 3068 | - | Correction of applicability for Redirection to UTRAN FDD/TDD test cases | 18.6.0 | 18.7.0 |  |
| 2024-12 | RAN#106 | R5-247287 | 3069 | - | Corrections to IoT NTN DCQR test case 13.6.2.4 | 18.6.0 | 18.7.0 |  |
| 2024-12 | RAN#106 | R5-247288 | 3070 | - | Corrections to IoT NTN DCQR test cases in RRC Connected | 18.6.0 | 18.7.0 |  |
| 2024-12 | RAN#106 | R5-247788 | 3061 | 1 | Addition of new test case 13.3.1.1_1 for NB IoT NTN | 18.6.0 | 18.7.0 |  |
| 2024-12 | RAN#106 | R5-247789 | 3062 | 1 | Addition of new test case 13.1.1.2_1 for NB IoT NTN | 18.6.0 | 18.7.0 |  |
| 2024-12 | RAN#106 | R5-247790 | 3059 | 1 | Update of test configuration for NTN in Clause 13.1.1 | 18.6.0 | 18.7.0 |  |
| 2024-12 | RAN#106 | R5-247791 | 3067 | 1 | test time updates to RRM TC 13.6.2.1 | 18.6.0 | 18.7.0 |  |
| 2024-12 | RAN#106 | R5-247799 | 3063 | 1 | Update of test configuration for NTN in Clause 13.3.1 | 18.6.0 | 18.7.0 |  |
| 2025-03 | RAN#107 | R5-250034 | 3072 | - | Update Annex E and Annex F for NB-IoT NTN | 18.7.0 | 18.8.0 |  |
| 2025-03 | RAN#107 | R5-250035 | 3073 | - | Update of q-QualMin-r13 to NB-IoT NTN RRM enhanced coverage TCs | 18.7.0 | 18.8.0 |  |
| 2025-03 | RAN#107 | R5-250036 | 3074 | - | Update of test configuration to NB-IoT NTN RRM enhanced coverage TCs | 18.7.0 | 18.8.0 |  |
| 2025-03 | RAN#107 | R5-250044 | 3075 | - | Addition of new test case 13.1.1.4 for NB IoT NTN | 18.7.0 | 18.8.0 |  |
| 2025-03 | RAN#107 | R5-250046 | 3077 | - | Addition of new test case 13.1.1.6 for NB IoT NTN | 18.7.0 | 18.8.0 |  |
| 2025-03 | RAN#107 | R5-250062 | 3078 | - | Editorial corrections of NB-IoT NTN RLM test cases | 18.7.0 | 18.8.0 |  |
| 2025-03 | RAN#107 | R5-250667 | 3079 | - | Core alignment in test case HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode TC | 18.7.0 | 18.8.0 |  |
| 2025-03 | RAN#107 | R5-250917 | 3082 | - | Corrections to IoT NTN test case 13.6.2.3 | 18.7.0 | 18.8.0 |  |
| 2025-03 | RAN#107 | R5-251567 | 3083 | 1 | Corrections to NPDSCH Repetition for IoT NTN DCQR test cases under enhanced coverage | 18.7.0 | 18.8.0 |  |
| 2025-03 | RAN#107 | R5-251569 | 3071 | 1 | Addition of new test case 13.3.1.3 for NB IoT NTN | 18.7.0 | 18.8.0 |  |
| 2025-03 | RAN#107 | R5-251570 | 3076 | 1 | Addition of new test case 13.1.1.5 for NB IoT NTN | 18.7.0 | 18.8.0 |  |
| 2025-03 | RAN#107 | R5-251572 | 3080 | 1 | Add TT analysis result (=0) to NB-IoT ReEST test cases : 13.3.1.1 and 13.3.1.2 | 18.7.0 | 18.8.0 |  |
| 2025-03 | RAN#107 | R5-251694 | 3081 | 1 | NPDCCH and NPDSCH repetition for nb-ntn DCQR and random access test cases in enhanced coverage | 18.7.0 | 18.8.0 |  |
| 2025-06 | RAN#108 | R5-251828 | 3085 | - | Update Editor notes and Add TT analysis result to NB-IoT ReEST test cases : 13.3.1.3 | 18.8.0 | 18.9.0 |  |
| 2025-06 | RAN#108 | R5-252126 | 3089 | - | Update Annex F for IoT NTN enh TCs based on MUTT analysis | 18.8.0 | 18.9.0 |  |
| 2025-06 | RAN#108 | R5-252165 | 3090 | - | Update to IoT NTN Nprach and DCQR TCs | 18.8.0 | 18.9.0 |  |
| 2025-06 | RAN#108 | R5-252888 | 3093 | - | Corrections to message contents for IoT NTN DCQR test case 13.6.2.3 | 18.8.0 | 18.9.0 |  |
| 2025-06 | RAN#108 | R5-253479 | 3087 | 1 | Update to IoT NTN enh multi cell TCs | 18.8.0 | 18.9.0 |  |
| 2025-06 | RAN#108 | R5-253484 | 3088 | 1 | Update Annex E for Iot NTN enh new TCs | 18.8.0 | 18.9.0 |  |
| 2025-06 | RAN#108 | R5-253593 | 3092 | 1 | Applicability updates to L2W and L2G reselection cases. | 18.8.0 | 18.9.0 |  |
| 2025-06 | RAN#108 | R5-253600 | 3086 | 1 | Update to test description of IoT NTN enh RRM TC 13.5.3 | 18.8.0 | 18.9.0 |  |
| 2025-09 | RAN#109 | R5-253721 | 3094 | - | Update test procedure for IoT NTN TC 13.4.1.2 and 13.4.1.3 | 18.9.0 | 18.10.0 |  |  |
| 2025-09 | RAN#109 | R5-253770 | 3095 | - | Update to random access test cases for IoT NTN test | 18.9.0 | 18.10.0 |  |  |
| 2025-09 | RAN#109 | R5-253771 | 3096 | - | Update to location-based cell reselection for IoT NTN enhancement | 18.9.0 | 18.10.0 |  |  |
| 2025-09 | RAN#109 | R5-253774 | 3097 | - | Update IoT NTN enh TCs including TT | 18.9.0 | 18.10.0 |  |  |
| 2025-09 | RAN#109 | R5-255344 | 3100 | 1 | Correction to LTE Cat 1bis RRM test case 5.1.19 and 5.1.20 | 18.9.0 | 18.10.0 |  |  |
| 2025-09 | RAN#109 | R5-255413 | 3102 | 1 | Correction to LTE measurement reporting test case 8.1.3 | 18.9.0 | 18.10.0 |  |  |
| 2025-09 | RAN#109 | R5-255419 | 3103 | 1 | Test Tolerance update for HD-FDD UE Timing Advance Adjustment Accuracy Test 7.2.9,13.4.2.1 and 13.4.2.2. | 18.9.0 | 18.10.0 |  |  |
| 2025-12 | RAN#110 | R5-255564 | 3105 | - | Update MUTT of IoT NTN TC 13.1.1.5 | 18.10.0 | 18.11.0 |  |  |
| 2025-12 | RAN#110 | R5-255588 | 3106 | - | Update test procedure and requirement of IoT NTN multi cell TCs | 18.10.0 | 18.11.0 |  |  |
| 2025-12 | RAN#110 | R5-255589 | 3107 | - | Editorial correction to NB IoT RRM TC | 18.10.0 | 18.11.0 |  |  |
| 2025-12 | RAN#110 | R5-256040 | 3110 | - | Update to Annex F MUTT for IoT NTN RRM TCs | 18.10.0 | 18.11.0 |  |  |
| 2025-12 | RAN#110 | R5-256041 | 3111 | - | Update to IoT NTN RRM TCs | 18.10.0 | 18.11.0 |  |  |
| 2025-12 | RAN#110 | R5-256857 | 3108 | 1 | Corrections for RRM HD-FDD UE Timing Advance Adjustment Accuracy Tests including TT | 18.10.0 | 18.11.0 |  |  |