| 3GPP TS 38.133 V17.10.0 (2023-06) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for support of radio resource management (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	82

1	Scope	84

2	References	84

3	Definitions, symbols and abbreviations	85

3.1	Definitions	85

3.2	Symbols	86

3.3	Abbreviations	87

3.4	Test tolerances	90

3.5	Frequency bands grouping	90

3.5.1	Introduction	90

3.5.2	NR operating bands in FR1	90

3.5.2A	NR operating bands for satellite access in FR1	91

3.5.3	NR operating bands in FR2	91

3.6	Applicability of requirements in this specification version	92

3.6.1	RRC connected state requirements in DRX	93

3.6.2	Number of serving carriers	93

3.6.2.1	Number of serving carriers for SA	93

3.6.2.2	Number of serving carriers for EN-DC	93

3.6.2.3	Number of serving carriers for NE-DC	93

3.6.2.4	Number of serving carriers for NR-DC	94

3.6.3	Applicability for intra-band FR2	94

3.6.4	Applicability for FR2 UE power classes	94

3.6.5	Applicability for SDL bands	94

3.6.6	Applicability of requirements for NGEN-DC operation	94

3.6.7	Applicability of QCL	94

3.6.9	Applicability of requirements for scheduling availability	95

3.6.10	Applicability of requirements for measurement restrictions	95

3.6.11	Applicability of requirements for Redcap UEs	95

3.6.11.1	RRC connected state requirements in DRX	95

3.6.11.2	Applicability for FR2 Redcap UE power classes	95

3.6.11.3	Applicability of QCL	95

3.6.12	Applicability of requirements for Satellite Access	96

3.6.13	Applicability of requirements for FR2	96

3.6.14	Applicability of requirements for FR2 Power Class 6	96

4	SA: RRC_IDLE state mobility	96

4.1	Cell Selection	96

4.2	Cell Re-selection	96

4.2.1	Introduction	96

4.2.2	Requirements	97

4.2.2.1	UE measurement capability	97

4.2.2.2	Measurement and evaluation of serving cell	97

4.2.2.3	Measurements of intra-frequency NR cells	98

4.2.2.4	Measurements of inter-frequency NR cells	100

4.2.2.5	Measurements of inter-RAT E-UTRAN cells	102

4.2.2.6	Maximum interruption in paging reception	104

4.2.2.7	General requirements	104

4.2.2.8	Minimum requirement at transitions	104

4.2.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	105

4.2.2.9.1	Introduction	105

4.2.2.9.2	Measurements for UE fulfilling low mobility criterion	105

4.2.2.9.3	Measurements for UE fulfilling not-at-cell edge criterion	106

4.2.2.9.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	107

4.2.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	107

4.2.2.10.1	Introduction	107

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion	107

4.2.2.10.3	Measurements for UE fulfilling not-at-cell edge criterion	108

4.2.2.10.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	109

4.2.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	110

4.2.2.11.1	Introduction	110

4.2.2.11.2	Measurements for UE fulfilling low mobility criterion	110

4.2.2.11.3	Measurements for UE fulfilling with not-at-cell edge criterion	111

4.2.2.11.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	112

4.2A	Cell Re-selection when subject to CCA	112

4.2A.1	Introduction	112

4.2A.2	Requirements	113

4.2A.2.1	UE measurement capability	113

4.2A.2.2	Measurement and evaluation when subject to CCA on the serving cell	113

4.2A.2.3	Measurements of intra-frequency NR cells when subject to CCA on the serving cell and target cell	114

4.2A.2.4	Measurements of inter-frequency NR cells when subject to CCA on the target cell	115

4.2A.2.5	Measurements of inter-RAT E-UTRAN cells when subject to CCA on the serving cell	117

4.2A.2.6	Maximum interruption in paging reception when subject to CCA on the target cell	117

4.2A.2.7	General requirements	118

4.2B	Cell Re-selection for RedCap	118

4.2B.1	Introduction	118

4.2B.2	Requirements	118

4.2B.2.1	UE measurement capability for RedCap	118

4.2B.2.1.1	UE measurement capability for 1 Rx RedCap	118

4.2B.2.1.2	UE measurement capability for 2 Rx RedCap	118

4.2B.2.2	Measurement and evaluation of serving cell for RedCap UE	118

4.2B.2.3	Measurements of intra-frequency NR cells for RedCap UE	121

4.2B.2.4	Measurements of inter-frequency NR cells for RedCap UE	123

4.2B.2.5	Measurements of inter-RAT E-UTRAN cells for RedCap UE	126

4.2B.2.6	Maximum interruption in paging reception for RedCap	128

4.2B.2.7	General requirements for RedCap	128

4.2B.2.8	Minimum requirement at transitions	129

4.2B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion for RedCap	129

4.2B.2.9.1	Introduction	129

4.2B.2.9.2	Measurements for UE fulfilling stationary criterion	130

4.2B.2.9.3	Measurements for a UE fulfilling stationary and not-at-cell edge criteria	133

4.2B.2.9.3A	Measurements for a UE fulfilling stationary and Rel-16 not at cell edge criteria	133

4.2B.2.9.4	Measurements for a UE fulfilling low mobility and stationary criteria	134

4.2B.2.9.5	Measurements for a UE fulfilling low mobility criterion and stationary and not-at-cell edge criteria	134

4.2B.2.9.6	Measurements for a UE fulfilling not-at-cell edge criterion and stationary and not-at-cell edge criteria	134

4.2B.2.9.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and stationary and notatcell edge criteria	134

4.2B.2.9.8	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and stationary criterion	135

4.2B.2.9.9	Measurements for UE fulfilling low mobility criterion	135

4.2B.2.9.10	Measurements for UE fulfilling not-at-cell edge criterion	138

4.2B.2.9.11	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	140

4.2B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	141

4.2B.2.10.1	Introduction	141

4.2B.2.10.2	Measurements for UE fulfilling stationary criterion	141

4.2B.2.10.3	Measurements for a UE fulfilling stationary not at cell edge criterion	144

4.2B.2.10.3A	Measurements for a UE fulfilling stationary and Rel-16 not at cell edge criterion	144

4.2B.2.10.4	Measurements for a UE fulfilling low mobility and stationary criteria	145

4.2B.2.10.5	Measurements for a UE fulfilling low mobility and stationary not at cell edge criteria	145

4.2B.2.10.6	Measurements for a UE fulfilling not-at-cell edge criterion and stationary not at cell edge criteria	145

4.2B.2.10.7	Measurements for a UE fulfilling low mobility not-at-cell edge criterion and stationary not at cell edge criteria	145

4.2B.2.10.8	Measurements for a UE fulfilling low mobility not-at-cell edge criterion and stationary criteria	146

4.2B.2.10.9	Measurements for UE fulfilling low mobility criterion	146

4.2B.2.10.10	Measurements for UE fulfilling not-at-cell edge criterion	149

4.2B.2.10.11	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	152

4.2B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	152

4.2B.2.11.1	Introduction	152

4.2B.2.11.2	Measurements for UE fulfilling stationary criterion	152

4.2B.2.11.3	Measurements for a UE fulfilling stationary not at cell edge criterion	154

4.2B.2.11.3A	Measurements for a UE fulfilling stationary and Rel-16 not at cell edge criterion	154

4.2B.2.11.4	Measurements for a UE fulfilling low mobility and stationary criteria	155

4.2B.2.11.5	Measurements for a UE fulfilling low mobility and stationary not at cell edge criteria	155

4.2B.2.11.6	Measurements for a UE fulfilling not-at-cell edge criterion and stationary not at cell edge criteria	155

4.2B.2.11.7	Measurements for a UE fulfilling low mobility not-at-cell edge criterion and stationary not at cell edge criteria	156

4.2B.2.11.8	Measurements for a UE fulfilling low mobility not-at-cell edge criterion and stationary criteria	156

4.2B.2.11.9	Measurements for UE fulfilling low mobility criterion	156

4.2B.2.11.10	Measurements for UE fulfilling with not-at-cell edge criterion	157

4.2B.2.11.11	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	159

4.2C	Cell Re-selection for NR UE for Satellite Access	159

4.2C.1	Introduction	159

4.2C.2	Requirements	159

4.2C.2.1	UE measurement capability	159

4.2C.2.2	Measurement and evaluation of serving cell	159

4.2C.2.3	Measurements of intra-frequency NR cells	160

4.2C.2.4	Measurements of inter-frequency NR cells	163

4.2C.2.5	Maximum interruption in paging reception	165

4.2C.2.6	Minimum requirement at transitions	166

4.2C.2.7	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	166

4.2C.2.8	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	166

4.2C.2.9	General requirements	166

4.3	Minimization of Drive Tests (MDT)	166

4.3.1	Introduction	166

4.3.2	Measurement Requirements	167

4.3.3	Requirements for Relative Time Stamp Accuracy	167

4.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	167

4.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	168

4.3C	Minimization of Drive Tests (MDT) for Satellite Access	168

4.3C.1	Introduction	168

4.3C.2	Measurement Requirements	168

4.3C.3	Requirements for Relative Time Stamp Accuracy	169

4.3C.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	169

4.3C.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	169

4.4	Idle Mode CA/DC Measurements	170

4.4.1	Introduction	170

4.4.2	Measurement Requirements	170

4.4.2.1	Detected cell requirement during state transition and Idle mode	170

4.4.2.2	Measurements of inter-frequency CA/DC candidate cells	170

4.4.2.3	Measurements on serving cell	172

4.4.2.4	Measurements of E-UTRAN inter-RAT DC candidate cells	172

5	SA: RRC_INACTIVE state mobility	173

5.1	Cell Re-selection	173

5.1.1	Introduction	173

5.1.2	Requirements	173

5.1.2.1	UE measurement capability	173

5.1.2.2	Measurement and evaluation of serving cell	173

5.1.2.3	Measurements of intra-frequency NR cells	173

5.1.2.4	Measurements of inter-frequency NR cells	173

5.1.2.5	Measurements of inter-RAT E-UTRAN cells	173

5.1.2.6	Maximum interruption in paging reception	173

5.1.2.7	General requirements	174

5.1A	Cell Re-selection with CCA	174

5.1A.1	Introduction	174

5.1A.2	Requirements	174

5.1A.2.1	UE measurement capability	174

5.1A.2.2	Measurement and evaluation when CCA is used on the serving cell	174

5.1A.2.3	Measurements of intra-frequency NR cells when CCA is used on the serving cell and target cell	174

5.1A.2.4	Measurements of inter-frequency NR cells when CCA is used on the target cell	174

5.1A.2.5	Measurements of inter-RAT E-UTRAN cells when CCA is used on the serving cell	174

5.1A.2.6	Maximum interruption in paging reception when CCA is used on the target cell	174

5.1A.2.7	General requirements	174

5.1B	Cell Re-selection for RedCap	175

5.1B.1	Introduction	175

5.1B.2	Requirements	175

5.1B.2.1	UE measurement capability	175

5.1B.2.2	Measurement and evaluation of serving cell	175

5.1B.2.3	Measurements of intra-frequency NR cells	176

5.1B.2.4	Measurements of inter-frequency NR cells	177

5.1B.2.5	Measurements of inter-RAT E-UTRAN cells	178

5.1B.2.6	Maximum interruption in paging reception	178

5.1B.2.7	General requirements	178

5.1B.2.8	Minimum requirement at transitions	178

5.1B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	178

5.1B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	179

5.1B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	179

5.2	Void	179

5.2B	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	179

5.2B.1	Introduction	179

5.2B.2	Requirements on UE synchronization for small data transmissions for RedCap	179

5.2B.2.2	Scheduling restriction	180

5.2B.2.3	Applicability conditions for SDT	180

5.3	Minimization of Drive Tests (MDT)	181

5.3.1	Introduction	181

5.3.2	Measurement Requirements	181

5.3.3	Requirements for Relative Time Stamp Accuracy	181

5.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	181

5.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	181

5.3.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	181

5.3C	Minimization of Drive Tests (MDT) for Satellite Access	181

5.3C.1	Introduction	182

5.3C.2	Measurement Requirements	182

5.3C.3	Requirements for Relative Time Stamp Accuracy	182

5.3C.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	182

5.3C.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	182

5.3C.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	182

5.4	Idle Mode CA/DC Measurements	183

5.4.1	Introduction	183

5.4.2	Measurement Requirements	183

5.4.2.1	Detected cell requirement during state transition and Idle mode	183

5.4.2.2	Measurements of inter-frequency CA/DC candidate cells	183

5.4.2.3	Measurements on serving cell	183

5.4.2.4	Measurements on E-UTRAN inter-RAT DC candidate cells	183

5.5	Configured Grant based Small Data Transmissions (CG-SDT)	183

5.5.1	Introduction	183

5.5.2	Requirements on UE synchronization for small data transmissions	183

5.5.3	TA validation requirements	184

5.5.4	Scheduling restriction	184

5.5.4.1	Scheduling availability of UE performing measurements in TDD bands on FR1	184

5.5.4.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	185

5.5.4.3	Scheduling availability of UE performing measurements on FR2	185

5.5.5	Applicability conditions for SDT	185

5.6	NR measurements for positioning	186

5.6.1	Introduction	186

5.6.2	RSTD measurements	186

5.6.2.1	Introduction	186

5.6.2.2	Requirements Applicability	186

5.6.2.3	Measurement Capability	187

5.6.2.5	Measurements Period Requirements	187

5.6.3	PRS-RSRP measurements	190

5.6.3.1	Introduction	190

5.6.3.2	Requirements applicability	190

5.6.3.3	Measurement Capability	190

5.6.3.4	Measurement Reporting Requirements	190

5.6.3.5	Measurement Period Requirements	191

5.6.4	UE Rx-Tx time difference measurements	193

5.6.4.1	Introduction	193

5.6.4.2	Requirements Applicability	193

5.6.4.3	Measurement Capability	193

5.6.4.4	Measurement Reporting Requirements	193

5.6.4.5	Measurement Period Requirements	194

5.6.5	PRS-RSRPP measurements	196

5.6.5.1	Introduction	196

5.6.5.2	Requirements applicability	196

5.6.5.3	Measurement capability	196

5.6.5.4	Measurement reporting requirements	196

5.6.5.5	Measurement period requirements	197

5.7	Random access based Small Data Transmissions (RA-SDT)	197

5.7.1	Introduction	197

5.7.2	Requirements for small data transmissions based on 2-step RA	197

5.7.3	Requirements for small data transmissions based on 4-step RA	197

6	RRC_CONNECTED state mobility	198

6.1	Handover	198

6.1.1	NR Handover	198

6.1.1.1	Introduction	198

6.1.1.2	NR FR1 - NR FR1 Handover	198

6.1.1.2.1	Handover delay	198

6.1.1.2.2	Interruption time	198

6.1.1.3	NR FR2- NR FR1 Handover	199

6.1.1.3.1	Handover delay	199

6.1.1.3.2	Interruption time	199

6.1.1.4	NR FR2- NR FR2 Handover	200

6.1.1.4.1	Handover delay	200

6.1.1.4.2	Interruption time	200

6.1.1.5	NR FR1- NR FR2 Handover	201

6.1.1.5.1	Handover delay	201

6.1.1.5.2	Interruption time	201

6.1.2	NR Handover to other RATs	202

6.1.2.1	NR – E-UTRAN Handover	202

6.1.2.1.1	Introduction	202

6.1.2.1.2	Handover delay	202

6.1.2.1.3	Interruption time	202

6.1.2.2	NR – UTRAN Handover	202

6.1.2.2.1	Introduction	202

6.1.2.2.2	Handover delay	202

6.1.2.2.3	Interruption time	203

6.1.3	NR DAPS Handover	203

6.1.3.1	Introduction	203

6.1.3.2	NR FR1 - NR FR1 DAPS Handover	204

6.1.3.2.1	DAPS handover delay	204

6.1.3.2.2	Interruption time	205

6.1.3.3	NR FR2- NR FR1 DAPS Handover	206

6.1.3.3.1	DAPS handover delay	207

6.1.3.3.2	Interruption time	207

6.1.3.4	NR FR1- NR FR2 DAPS Handover	208

6.1.3.4.1	DAPS handover delay	208

6.1.3.4.2	Interruption time	208

6.1.4	NR Conditional Handover	209

6.1.4.1	Introduction	209

6.1.4.2	NR FR1 – NR FR1 conditional handover	209

6.1.4.3	NR FR2 – NR FR1 conditional handover	210

6.1.4.4	NR FR2 – NR FR2 conditional handover	210

6.1.4.4.1	Handover delay	210

6.1.4.4.2	Measurement time	211

6.1.4.4.3	Preparation time	211

6.1.4.4.4	Interruption time	211

6.1.4.5	NR FR1 – NR FR2 conditional handover	212

6.1.5	NR Handover with PSCell	212

6.1.5.1	Introduction	212

6.1.5.2	Handover with PSCell from NR SA to EN-DC	212

6.1.5.2.1	Interruption time for inter-RAT HO from NR to E-UTRAN	212

6.1.5.2.2	PSCell addition in HO with PSCell for NR SA to EN-DC	213

6.1.5.3	HO with PSCell from NE-DC to NE-DC	213

6.1.5.3.1	Handover delay	213

6.1.5.3.2	HO with PSCell - PCell Interruption time	214

6.1.5.3.3	PSCell addition/change in NE-DC to NE-DC HO with PSCell	214

6.1.5.4	HO with PSCell from NR-DC to NR-DC	214

6.1.5.5	Handover with PSCell from NR SA to EN-DC with PSCell using CCA	215

6.1.5.5.1	Introduction	215

6.1.5.5.2	NR SA to EN-DC HO with PSCell- NR to E-UTRA HO Interruption time	216

6.1.5.5.3	NR SA to EN-DC HO with PSCell - NR PSCell Addition Delay requirements	216

6.1A	Void	217

6.1A.1	Void	217

6.1A.1.1	Void	217

6.1A.1.2	Void	217

6.1A.1.2.1	Void	217

6.1A.1.2.2	Void	217

6.1B	Handover to target cell using CCA	217

6.1B.1	NR Handover	217

6.1B.1.1	Introduction	217

6.1B.1.2	NR FR1 - NR FR1 Handover	218

6.1B.1.2.1	Handover delay	218

6.1B.1.2.2	Interruption time	218

6.1B.1.3	NR FR2-2 NR FR2-2 Handover	219

6.1B.1.3.1	Handover delay	219

6.1B.1.3.2	Interruption time	219

6.1B.1.4	NR FR1- NR FR2-2 Handover	220

6.1B.1.4.1	Handover delay	220

6.1B.1.4.2	Interruption time	220

6.1C	Handover for SAN	221

6.1C.1	NR SAN Handover	221

6.1C.1.1	Introduction	221

6.1C.1.2	NR SAN FR1 – NR SAN FR1 Handover	221

6.1C.1.2.1	Handover delay	221

6.1C.1.2.2	Interruption time	221

6.1C.2	NR SAN Conditional Handover	222

6.1C.2.1	Introduction	222

6.1C.2.2	NR SAN FR1 – NR SAN FR1 conditional handover	222

6.1C.2.2.1	Handover delay	222

6.1C.2.2.2	Measurement time	223

6.1C.2.2.3	Preparation time	224

6.1C.2.2.4	Interruption time	224

6.1D	Handover for RedCap	224

6.1D.1	NR Handover	224

6.1D.1.1	Introduction	224

6.1D.1.2	NR FR1 - NR FR1 Handover	225

6.1D.1.3	NR FR2- NR FR2 Handover	225

6.1D.2	NR Handover to other RATs	225

6.1D.2.1	NR – E-UTRAN Handover	225

6.2	RRC Connection Mobility Control	226

6.2.1	SA: RRC Re-establishment	226

6.2.1.1	Introduction	226

6.2.1.2	Requirements	226

6.2.1.2.1	UE Re-establishment delay requirement	226

6.2.1A	RRC Re-establishment with CCA	228

6.2.1A.1	Introduction	228

6.2.1A.2	Requirements	228

6.2.1A.2.1	UE Re-establishment with CCA delay requirement	228

6.2.1B	SA: RRC Re-establishment for RedCap	230

6.2.1B.1	Introduction	230

6.2.1B.2	Requirements	230

6.2.2	Random access	230

6.2.2.1	Introduction	230

6.2.2.2	Requirements for 4-step RA type	231

6.2.2.2.1	Contention based random access	231

6.2.2.2.1.1	Correct behaviour when transmitting Random Access Preamble	231

6.2.2.2.1.2	Correct behaviour when receiving Random Access Response	231

6.2.2.2.1.3	Correct behaviour when not receiving Random Access Response	231

6.2.2.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	231

6.2.2.2.1.5	SA: Correct behaviour when receiving a message over Temporary C-RNTI	232

6.2.2.2.1.6	Correct behaviour when contention Resolution timer expires	232

6.2.2.2.2	Non-Contention based random access	232

6.2.2.2.2.1	Correct behaviour when transmitting Random Access Preamble	232

6.2.2.2.2.2	Correct behaviour when receiving Random Access Response	232

6.2.2.2.2.3	Correct behaviour when not receiving Random Access Response	233

6.2.2.2.3	UE behaviour when configured with supplementary UL	233

6.2.2.3	Requirements for 2-step RA type	233

6.2.2.3.1	Contention based random access	233

6.2.2.3.1.1	Correct behaviour when transmitting MsgA	233

6.2.2.3.1.2	Correct behaviour when receiving MsgB	234

6.2.2.3.1.3	Correct behaviour when not receiving MsgB	234

6.2.2.3.2	Non-Contention based random access	234

6.2.2.3.2.1	Correct behaviour when transmitting MsgA	234

6.2.2.3.2.2	Correct behaviour when receiving MsgB	234

6.2.2.3.2.3	Correct behaviour when not receiving MsgB	234

6.2.2.3.3	UE behaviour when configured with supplementary UL	235

6.2.2A	Random access when CCA is used on target frequency	235

6.2.2A.1	Introduction	235

6.2.2A.2	Requirements for 4-step RA type	235

6.2.2A.2.1	Contention based random access	235

6.2.2A.2.1.1	Correct behaviour when transmitting Random Access Preamble	235

6.2.2A.2.1.2	Correct behaviour when receiving Random Access Response	236

6.2.2A.2.1.3	Correct behaviour when not receiving Random Access Response	236

6.2.2A.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	236

6.2.2A.2.1.6	Correct behaviour when contention Resolution timer expires	236

6.2.2A.2.2	Non-Contention based random access	236

6.2.2A.2.2.1	Correct behaviour when transmitting Random Access Preamble	236

6.2.2A.2.2.2	Correct behaviour when receiving Random Access Response	237

6.2.2A.2.2.3	Correct behaviour when not receiving Random Access Response	237

6.2.2A.3	Requirements for 2-step RA type	237

6.2.2A.3.1	Contention based random access	238

6.2.2A.3.1.1	Correct behaviour when transmitting MsgA	238

6.2.2A.3.1.2	Correct behaviour when receiving MsgB	238

6.2.2A.3.1.3	Correct behaviour when not receiving MsgB	239

6.2.2A.3.2	Non-Contention based random access	239

6.2.2A.3.2.1	Correct behaviour when transmitting MsgA	239

6.2.2A.3.2.2	Correct behaviour when receiving MsgB	239

6.2.2A.3.2.3	Correct behaviour when not receiving MsgB	239

6.2.2B	Random access for RedCap	240

6.2.2B.1	Introduction	240

6.2.2B.2	Requirements	240

6.2.3	SA: RRC Connection Release with Redirection	240

6.2.3.1	Introduction	240

6.2.3.2	Requirements	241

6.2.3.2.1	RRC connection release with redirection to NR	241

6.2.3.2.2	RRC connection release with redirection to E-UTRAN	241

6.2.3.2.3	RRC connection release with redirection to NR carrier subject to CCA	242

6.2.3A	SA: RRC Connection Release with Redirection for RedCap	243

6.2.3A.1	Introduction	243

6.2.3A.2	Requirements	243

6.2.3A.2.1	RRC connection release with redirection to NR	243

6.2.3A.2.2	RRC connection release with redirection to E-UTRAN	244

6.2C	RRC Connection Mobility Control for Satellite Access	244

6.2C.1	SA: RRC Re-establishment for Satellite Access	244

6.2C.1.1	Introduction	244

6.2C.1.2	Requirements	244

6.2C.1.2.1	UE Re-establishment delay requirement	244

6.2C.2	Random access for satellite access	245

6.2C.2.1	Introduction	245

6.2C.2.2	Requirements for 4-step RA type	246

6.2C.2.2.1	Contention based random access	246

6.2C.2.2.1.1	Correct behaviour when transmitting Random Access Preamble	246

6.2C.2.2.1.2	Correct behaviour when receiving Random Access Response	246

6.2C.2.2.1.3	Correct behaviour when not receiving Random Access Response	246

6.2C.2.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	246

6.2C.2.2.1.5	SA: Correct behaviour when receiving a message over Temporary C-RNTI	246

6.2C.2.2.1.6	Correct behaviour when contention Resolution timer expires	247

6.2C.2.2.2	Non-Contention based random access	247

6.2C.2.2.2.1	Correct behaviour when transmitting Random Access Preamble	247

6.2C.2.2.2.2	Correct behaviour when receiving Random Access Response	247

6.2C.2.2.2.3	Correct behaviour when not receiving Random Access Response	248

6.2C.2.3	Requirements for 2-step RA type	248

6.2C.2.3.1	Contention based random access	248

6.2C.2.3.1.1	Correct behaviour when transmitting MsgA	248

6.2C.2.3.1.2	Correct behaviour when receiving MsgB	248

6.2C.2.3.1.3	Correct behaviour when not receiving MsgB	249

6.2C.2.3.2	Non-Contention based random access	249

6.2C.2.3.2.1	Correct behaviour when transmitting MsgA	249

6.2C.2.3.2.2	Correct behaviour when receiving MsgB	249

6.2C.2.3.2.3	Correct behaviour when not receiving MsgB	249

6.2C.3	SA: RRC Connection Release with Redirection for Satellite Access	249

6.2C.3.1	Introduction	249

6.2C.3.2	Requirements	250

6.2C.3.2.1	RRC connection release with redirection to NR	250

7	Timing	251

7.1	UE transmit timing	251

7.1.1	Introduction	251

7.1.2	Requirements	251

7.1.2.1	Gradual timing adjustment	253

7.1.2.2	Void	253

7.1.2.3	One shot large UL timing adjustment for FR2 Power Class 6 UE	253

7.1A	UE transmit timing for RedCap	254

7.1A.1	Introduction	254

7.1A.2	Requirements	254

7.1A.2.1	Gradual timing adjustment	255

7.1C	UE transmit timing for Satellite Access	256

7.1C.1	Introduction	256

7.1C.2	Requirements	256

7.1C.2.1	Gradual timing adjustment	257

7.2	UE timer accuracy	257

7.2.1	Introduction	257

7.2.2	Requirements	257

7.2A	UE timer accuracy for RedCap	258

7.2A.1	Introduction	258

7.2A.2	Requirements	258

7.2C	UE timer accuracy for satellite access	258

7.2C.1	Introduction	258

7.2C.2	Requirements	258

7.3	Timing advance	259

7.3.1	Introduction	259

7.3.2	Requirements	259

7.3.2.1	Timing Advance adjustment delay	259

7.3.2.2	Timing Advance adjustment accuracy	259

7.3A	Timing Advance for RedCap	259

7.3A.1	Introduction	259

7.3A.2	Requirements	259

7.3A.2.1	Timing Advance adjustment delay	259

7.3A.2.2	Timing Advance adjustment accuracy	259

7.3C	Timing advance for satellite access	260

7.3C.1	Introduction	260

7.3C.2	Requirements	260

7.3C.2.1	Timing Advance adjustment delay	260

7.3C.2.2	Timing Advance adjustment accuracy	260

7.4	Cell phase synchronization accuracy	260

7.4.1	Definition	260

7.4.2	Minimum requirements	260

7.5	Maximum Transmission Timing Difference	261

7.5.1	Introduction	261

7.5.2	Minimum Requirements for inter-band EN-DC	261

7.5.2.1	Minimum Requirements for inter-band synchronous EN-DC	261

7.5.3	Minimum Requirements for intra-band EN-DC	262

7.5.4	Minimum Requirements for NR Carrier Aggregation	262

7.5.5	Minimum Requirements for inter-band NE-DC	263

7.5.5.1	Minimum Requirements for inter-band synchronous NE-DC	263

7.5.6	Minimum Requirements for inter-band NR DC	263

7.6	Maximum Receive Timing Difference	264

7.6.1	Introduction	264

7.6.2	Minimum Requirements for inter-band EN-DC	265

7.6.2.1	Minimum Requirements for inter-band synchronous EN-DC	265

7.6.3	Minimum Requirements for intra-band EN-DC	265

7.6.4	Minimum Requirements for NR Carrier Aggregation	266

7.6.5	Minimum Requirements for inter-band NE-DC	266

7.6.5.1	Minimum Requirements for inter-band synchronous NE-DC	267

7.6.6	Minimum Requirements for inter-band NR DC	267

7.7	deriveSSB-IndexFromCell tolerance	268

7.7.1	Minimum requirements	268

7.7A	deriveSSB-IndexFromCell tolerance for RedCap	269

7.7A.1	Minimum requirements	269

7.8	Void	269

7.9	deriveSSB-IndexFromCellInter-r17 tolerance	269

7.9.1	Minimum requirements	269

8	Signalling characteristics	255

8.1	Radio Link Monitoring	255

8.1.1	Introduction	255

8.1.1.1	Introduction of Requirement on Radio Link Monitoring for UE Configured with Relaxed Measurement Criteria	256

8.1.2	Requirements for SSB based radio link monitoring	256

8.1.2.1	Introduction	257

8.1.2.2	Minimum requirement	257

8.1.2.3	Measurement restrictions for SSB based RLM	261

8.1.2.4	Minimum requirement of SSB based radio link monitoring for UE fulfilling relaxed measurement criteria	261

8.1.3	Requirements for CSI-RS based radio link monitoring	262

8.1.3.1	Introduction	262

8.1.3.2	Minimum requirement	263

8.1.3.3	Measurement restrictions for CSI-RS based RLM	266

8.1.3.4	Minimum requirement of CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria	267

8.1.4	Minimum requirement at transitions	268

8.1.5	Minimum requirement for UE turning off the transmitter	268

8.1.6	Minimum requirement for L1 indication	268

8.1.7	Scheduling availability of UE during radio link monitoring	269

8.1.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	269

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	269

8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2	269

8.1.7.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	270

8.1A	Radio Link Monitoring with CCA on Target Frequency	270

8.1A.1	Introduction	270

8.1A.2	Requirements for SSB Based Radio Link Monitoring	271

8.1A.2.1	Introduction	271

8.1A.2.2	Minimum Requirement	272

8.1A.3	Minimum requirement at transitions	275

8.1A.4	Minimum requirement for UE turning off the transmitter	276

8.1A.5	Minimum requirement for L1 indication	276

8.1A.6	Scheduling availability of UE during radio link monitoring	276

8.1A.6.3	Scheduling availability of UE performing radio link monitoring on FR2-2	276

8.1A.6.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	277

8.1B	Radio Link Monitoring for RedCap	277

8.1B.1	Introduction	277

8.1B.2	Requirements for SSB based radio link monitoring	278

8.1B.2.1	Introduction	278

8.1B.2.2	Minimum requirement	279

8.1B.2.3	Measurement restrictions for SSB based RLM	281

8.1B.3	Requirements for CSI-RS based radio link monitoring	282

8.1B.3.1	Introduction	282

8.1B.3.2	Minimum requirement	282

8.1B.3.3	Measurement restrictions for CSI-RS based RLM	285

8.1B.4	Minimum requirement at transitions	286

8.1B.5	Minimum requirement for UE turning off the transmitter	286

8.1B.6	Minimum requirement for L1 indication	286

8.1B.7	Scheduling availability of UE during radio link monitoring	286

8.1B.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	287

8.1B.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	287

8.1B.7.3	Scheduling availability of UE performing radio link monitoring on FR2	287

8.1C	Radio Link Monitoring for Satellite Access	287

8.1C.1	Introduction	287

8.1C.2	Requirements for SSB based radio link monitoring	288

8.1C.2.1	Introduction	288

8.1C.2.2	Minimum requirement	289

8.1C.2.3	Measurement restrictions for SSB based RLM	290

8.1C.3	Requirements for CSI-RS based radio link monitoring	290

8.1C.3.1	Introduction	290

8.1C.3.2	Minimum requirement	291

8.1C.3.3	Measurement restrictions for CSI-RS based RLM	292

8.1C.4	Minimum requirement at transitions	293

8.1C.5	Minimum requirement for UE turning off the transmitter	293

8.1C.6	Minimum requirement for L1 indication	293

8.1C.7	Scheduling availability of UE during radio link monitoring	293

8.1C.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	294

8.1C.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	294

8.2	Interruption	294

8.2.1	EN-DC Interruption	294

8.2.1.1	Introduction	294

8.2.1.2	Requirements	295

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX	295

8.2.1.2.2	Interruptions at transitions from non-DRX to DRX	295

8.2.1.2.3	Interruptions at SCell addition/release	295

8.2.1.2.4	Interruptions at SCell activation/deactivation	296

8.2.1.2.5	Interruptions during measurements on SCC	298

8.2.1.2.5.1	Interruptions during measurements on deactivated NR SCC	298

8.2.1.2.5.2	Interruptions during measurements on deactivated E-UTRAN SCC	298

8.2.1.2.5.3	Interruptions during CQI measurements on dormant E-UTRAN SCell	298

8.2.1.2.5.4	Interruptions during RRM measurements on dormant E-UTRAN SCC	298

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration	299

8.2.1.2.7	Interruptions due to Active BWP switching Requirement	299

8.2.1.2.8	Interruptions at direct SCell activation and hibernation	300

8.2.1.2.8.1	Interruptions during direct SCell activation and hibernation of E-UTRA SCell	300

8.2.1.2.8.2	Interruptions during direct SCell activation	301

8.2.1.2.9	Interruptions at SCell hibernation	301

8.2.1.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	301

8.2.1.2.11	Interruptions due to UE-specific CBW change	301

8.2.1.2.12	Interruptions at NR SRS carrier based switching	301

8.2.1.2.13	Interruptions at E-UTRA SRS carrier based switching	303

8.2.1.2.14	DL Interruptions at switching between two uplink carriers	304

8.2.1.2.15	Interruptions due to SCell dormancy	304

8.2.1.2.15.1	Interruptions due to SCell dormancy switch	304

8.2.1.2.15.2	Interruptions due to CQI measurements during SCell dormancy	304

8.2.1.2.15.3	Interruptions due to RRM measurements during SCell dormancy	305

8.2.1.2.16	Interruptions when identifying CGI of an NR cell with autonomous gaps	305

8.2.1.2.17	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	305

8.2.1.2.18	Interruptions at NR SRS antenna port switching	306

8.2.1.2.19	Interruptions at fast SCell activation	307

8.2.1.2.20	Interruptions due to PUCCH SCell activation/deactivation	308

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation	308

8.2.2.1	Introduction	308

8.2.2.2	Requirements	309

8.2.2.2.1	Interruptions at SCell addition/release	309

8.2.2.2.2	Interruptions at SCell activation/deactivation	310

8.2.2.2.3	Interruptions during measurements on deactivated SCC	311

8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration	312

8.2.2.2.5	Interruptions due to Active BWP switching Requirement	312

8.2.2.2.6	Interruptions at inter-frequency SFTD measurement	313

8.2.2.2.7	Interruptions at SCell activation/deactivation with multiple downlink SCells	315

8.2.2.2.8	Interruptions due to UE-specific CBW change	315

8.2.2.2.9	Interruptions at NR SRS carrier based switching	315

8.2.2.2.10	DL Interruptions at UE switching between two uplink carriers	317

8.2.2.2.10A	DL Interruptions at UE switching between two uplink carriers with two transmit antenna connectors	317

8.2.2.2.10B	DL Interruptions at UE switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	318

8.2.2.2.10C	DL Interruptions at UE switching between two uplink bands with two transmit antenna connectors	318

8.2.2.2.11	Interruptions at direct SCell activation	318

8.2.2.2.12	Interruptions due to SCell dormancy	319

8.2.2.2.12.1	Interruptions due to SCell dormancy switch	319

8.2.2.2.12.2	Interruptions due to CQI measurements during SCell dormancy	319

8.2.2.2.12.3	Interruptions due to RRM measurements during SCell dormancy	319

8.2.2.2.13	Interruptions at transitions between active and non-active during DRX	319

8.2.2.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	319

8.2.2.2.15	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	320

8.2.2.2.16	Interruptions at NR SRS antenna port switching	320

8.2.2.2.17	Interruptions at fast SCell activation	321

8.2.2.2.18	Interruptions due to PUCCH SCell activation/deactivation	322

8.2.3	NE-DC Interruptions	322

8.2.3.1	Introduction	322

8.2.3.2	Requirements	323

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX	323

8.2.3.2.2	Interruptions at transitions from non-DRX to DRX	323

8.2.3.2.3	Interruptions at PSCell/SCell addition/release	323

8.2.3.2.4	Interruptions at SCell activation/deactivation	325

8.2.3.2.5	Interruptions during measurements on SCC	326

8.2.3.2.5.1	Interruptions during measurements on deactivated NR SCC	326

8.2.3.2.5.2	Interruptions during measurements on deactivated E-UTRAN SCC	326

8.2.3.2.5.3	Interruptions during CQI measurements on dormant E-UTRAN SCC	327

8.2.3.2.5.4	Interruptions during RRM measurements on dormant E-UTRAN SCC	327

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration	327

8.2.3.2.7	Interruptions due to Active BWP switching Requirement	328

8.2.3.2.8	Interruptions at direct SCell activation and hibernation	328

8.2.3.2.9	Interruptions at SCell hibernation	328

8.2.3.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	329

8.2.3.2.11	 Interruptions at NR SRS carrier based switching	329

8.2.3.2.12	 Interruptions at E-UTRA SRS carrier based switching	330

8.2.3.2.13	Interruptions due to SCell dormancy	331

8.2.3.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	332

8.2.3.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	332

8.2.3.2.17	Interruptions at fast SCell activation	334

8.2.3.2.18	Interruptions due to UE-specific CBW change	334

8.2.3.2.19	Interruptions due to PUCCH SCell activation/deactivation	335

8.2.4	NR-DC: Interruptions	335

8.2.4.1	Introduction	335

8.2.4.2	Requirements	336

8.2.4.2.1	Interruptions at PSCell/SCell addition/release	336

8.2.4.2.2	Interruptions at SCell activation/deactivation	337

8.2.4.2.3	Interruptions during measurements on SCC	338

8.2.4.2.4	Interruptions at UL carrier RRC reconfiguration	338

8.2.4.2.5	Interruptions due to Active BWP switching Requirement	339

8.2.4.2.6	Interruptions at transitions between active and non-active during DRX	339

8.2.4.2.7	Interruptions at transitions from non-DRX to DRX	339

8.2.4.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	340

8.2.4.2.9	Interruptions at NR SRS carrier based switching	340

8.2.4.2.10	Interruptions at direct SCell activation	341

8.2.4.2.11	Interruptions when identifying CGI of an NR cell with autonomous gaps	342

8.2.4.2.12	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	342

8.2.4.2.13	 Interruptions due to SCell dormancy	343

8.2.4.2.14	Interruptions at NR SRS antenna port switching	343

8.2.4.2.15	Interruptions at fast SCell activation	344

8.2.4.2.16	Interruptions at SCG activation/deactivation	345

8.2.4.2.17	Interruptions due to RRM measurements on deactivated SCG	345

8.2.4.2.18 Interruptions during RLM/BFD measurements on deactivated PScell	346

8.2.4.2.19	Interruptions due to UE-specific CBW change	346

8.2.4.2A	Void	346

8.2.4.2A.1	Void	346

8.2.4.2A.2	Void	346

8.2.4.2A.3	Void	346

8.3	SCell Activation and Deactivation Delay	346

8.3.1	Introduction	346

8.3.2	SCell Activation Delay Requirement for Deactivated SCell	346

8.3.3	SCell Deactivation Delay Requirement for Activated SCell	351

8.3.4	Direct SCell Activation at SCell addition	351

8.3.5	Direct SCell Activation at Handover	353

8.3.7	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	354

8.3.8	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	358

8.3.9	Direct SCell Activation of Multiple Downlink SCells at SCell addition	359

8.3.10	Direct SCell Activation of Multiple Downlink SCells at Handover	360

8.3.12	SCell Activation Delay Requirement for Deactivated PUCCH SCell	361

8.3.13	SCell activation delay Requirement for Deactivated PUCCH SCell with Multiple SCells	364

8.3.13.1	Introduction	364

8.3.14	SCell Deactivation Delay Requirement for Activated PUCCH SCell	366

8.3.15	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple Downlink SCells	366

8.3.16	Fast SCell Activation Delay Requirement for Deactivated SCell	367

8.3A	SCell Activation and Deactivation Delay in Carriers with CCA	369

8.3A.1	Introduction	369

8.3A.2	SCell Activation Delay Requirement for Deactivated SCell	370

8.3A.3	SCell Deactivation Delay Requirement for Activated SCell	374

8.4	UE UL carrier RRC reconfiguration delay	374

8.4.1	Introduction	374

8.4.2	UE UL carrier configuration delay requirement	374

8.4.3	UE UL carrier deconfiguration delay requirement	375

8.5	Link Recovery Procedures	375

8.5.1	Introduction	375

8.5.1.1	Introduction of Requirement on Link Recovery Procedures for UE configured with relaxed measurement criteria	376

8.5.2	Requirements for SSB based beam failure detection	377

8.5.2.1	Introduction	377

8.5.2.2	Minimum requirement	377

8.5.2.3	Measurement restriction for SSB based beam failure detection	380

8.5.2.4	Minimum requirement of SSB based beam failure detection for UE fulfilling relaxed measurement criteria	381

8.5.3	Requirements for CSI-RS based beam failure detection	382

8.5.3.1	Introduction	382

8.5.3.2	Minimum requirement	382

8.5.3.3	Measurement restrictions for CSI-RS beam failure detection	386

8.5.3.4	Minimum requirement of CSI-RS based beam failure detection for UE fulfilling relaxed measurement criteria	387

8.5.4	Minimum requirement for L1 indication	387

8.5.5	Requirements for SSB based candidate beam detection	388

8.5.5.1	Introduction	388

8.5.5.2	Minimum requirement	388

8.5.5.3	Measurement restriction for SSB based candidate beam detection	391

8.5.6	Requirements for CSI-RS based candidate beam detection	391

8.5.6.1	Introduction	391

8.5.6.2	Minimum requirement	392

8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection	395

8.5.7	Scheduling availability of UE during beam failure detection	395

8.5.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	395

8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	396

8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2	396

8.5.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	397

8.5.8	Scheduling availability of UE during candidate beam detection	397

8.5.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	397

8.5.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	397

8.5.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	397

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	398

8.5.9	Requirements for Beam Failure Recovery in SCell	398

8.5.9.1	Introduction	398

8.5.9.2	Requirement	398

8.5.10	Minimum requirement at transitions for beam failure detection	398

8.5A	Link Recovery Procedures when CCA is used on target frequency	399

8.5A.1	Introduction	399

8.5A.2	Requirements for SSB based beam failure detection	400

8.5A.2.1	Introduction	400

8.5A.2.2	Minimum requirement	400

8.5A.2.3	Measurement restriction for SSB based beam failure detection	402

8.5A.3	Void	403

8.5A.4	Minimum requirement for L1 indication	403

8.5A.5	Requirements for SSB based candidate beam detection	403

8.5A.5.1	Introduction	403

8.5A.5.2	Minimum requirement	403

8.5A.5.3	Measurement restriction for SSB based candidate beam detection	405

8.5A.6	Void	406

8.5A.7	Scheduling availability of UE during beam failure detection	406

8.5A.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH	406

8.5A.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH	406

8.5A.7.3	Scheduling availability of UE performing beam failure detection on FR2-2	406

8.5A.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR DC	406

8.5A.8	Scheduling availability of UE during candidate beam detection	406

8.5A.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2-2	407

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	407

8.5B	Link Recovery Procedures for Redcap	407

8.5B.1	Introduction	407

8.5B.2	Requirements for SSB based beam failure detection for Redcap	407

8.5B.2.1	Introduction	407

8.5B.2.2	Minimum requirement	408

8.5B.2.3	Measurement restriction for SSB based beam failure detection	409

8.5B.3	Requirements for CSI-RS based beam failure detection for Redcap	410

8.5B.3.1	Introduction	410

8.5B.3.2	Minimum requirement	410

8.5B.3.3	Measurement restrictions for CSI-RS beam failure detection	412

8.5B.4	Minimum requirement for L1 indication for Redcap	413

8.5B.5	Requirements for SSB based candidate beam detection for Redcap	414

8.5B.5.1	Introduction	414

8.5B.5.2	Minimum requirement	414

8.5B.5.3	Measurement restriction for SSB based candidate beam detection	416

8.5B.6	Requirements for CSI-RS based candidate beam detection for Redcap	416

8.5B.6.1	Introduction	416

8.5B.6.2	Minimum requirement	416

8.5B.6.3	Measurement restriction for CSI-RS based candidate beam detection	418

8.5B.7	Scheduling availability of UE during beam failure detection for Redcap	419

8.5B.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	419

8.5B.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	419

8.5B.7.3	Scheduling availability of UE performing beam failure detection on FR2	419

8.5B.8	Scheduling availability of UE during candidate beam detection for Redcap	420

8.5B.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	420

8.5B.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	420

8.5B.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	420

8.5B.9	Minimum requirement at transitions for beam failure detection for Redcap	420

8.5C	Link Recovery Procedures for Satellite Access	421

8.5C.1	Introduction	421

8.5C.2	Requirements for SSB based beam failure detection	421

8.5C.2.1	Introduction	421

8.5C.2.2	Minimum requirement	422

8.5C.2.3	Measurement restriction for SSB based beam failure detection	422

8.5C.3	Requirements for CSI-RS based beam failure detection	423

8.5C.3.1	Introduction	423

8.5C.3.2	Minimum requirement	423

8.5C.3.3	Measurement restrictions for CSI-RS beam failure detection	424

8.5C.4	Minimum requirement for L1 indication	425

8.5C.5	Requirements for SSB based candidate beam detection	425

8.5C.5.1	Introduction	425

8.5C.5.2	Minimum requirement	425

8.5C.5.3	Measurement restriction for SSB based candidate beam detection	426

8.5C.6	Requirements for CSI-RS based candidate beam detection	427

8.5C.6.1	Introduction	427

8.5C.6.2	Minimum requirement	427

8.5C.6.3	Measurement restriction for CSI-RS based candidate beam detection	428

8.5C.7	Scheduling availability of UE during beam failure detection	428

8.5C.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	428

8.5C.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	428

8.5C.8	Scheduling availability of UE during candidate beam detection	429

8.5C.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	429

8.5C.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	429

8.5C.9	Minimum requirement at transitions for beam failure detection	429

8.6	Active BWP switch delay	429

8.6.1	Introduction	429

8.6.2	DCI and timer based BWP switch delay on a single CC	430

8.6.2A	DCI based BWP switch delay on multiple CCs	431

8.6.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs	431

8.6.2A.2	Non-simultaneous DCI based BWP switch delay on multiple CCs	433

8.6.2B	Timer based BWP switch delay on multiple CCs	433

8.6.2B.1	Simultaneous timer based BWP switch delay on multiple CCs	433

8.6.2B.2	Non-simultaneous timer based BWP switch delay on multiple CCs	433

8.6.3	RRC based BWP switch delay on a single CC	434

8.6.3A	RRC based BWP switch delay on multiple CCs	435

8.6.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs	435

8.6.3A.2	Non-simultaneous RRC based BWP switch delay on multiple CCs	435

8.6.4	BWP switch delay on Consistent UL CCA recovery	436

8.6A	Active BWP switch delay for RedCap	436

8.6A.1	Introduction	436

8.6A.2	DCI and timer based BWP switch delay on a single CC	436

8.6A.3	RRC based BWP switch delay on a single CC	438

8.6C	Active BWP switch delay for satellite access	438

8.6C.1	Introduction	438

8.6C.2	DCI and timer based BWP switch delay on a single CC	438

8.6C.3	RRC based BWP switch delay on a single CC	440

8.7	Void	440

8.8	NE-DC: E-UTRAN PSCell Addition and Release Delay	440

8.8.1	Introduction	440

8.8.2	E-UTRAN PSCell Addition Delay Requirement	440

8.8.3	E-UTRAN PSCell Release Delay Requirement	441

8.9	NR-DC: PSCell Addition and Release Delay	441

8.9.1	Introduction	441

8.9.2	PSCell Addition Delay Requirement	441

8.9.3	PSCell Release Delay Requirement	442

8.9A	Conditional PSCell Addition Delay	442

8.9A.1	Introduction	442

8.9A.2	Conditional PSCell Addition Delay Requirement	442

8.9A.2.1	Measurement time	443

8.9B	NR-DC: PSCell Addition and Release Delay in Carriers with CCA	443

8.9B.1	Introduction	443

8.9B.2	PSCell Addition Delay Requirement	444

8.9B.3	PSCell Release Delay Requirement	444

8.10	Active TCI state switching delay	445

8.10.3A	MAC-CE based TCI state switch delay in HST FR2 scenarios	446

8.10.4	DCI based TCI state switch delay	446

8.10.5	RRC based TCI state switch delay	447

8.10.6	Active TCI state list update delay	447

8.10A	Active TCI state switching delay with CCA	447

8.10A.1	Introduction	447

8.10A.2	Known conditions for TCI state	448

8.10A.3	MAC-CE based TCI state switch delay	448

8.10A.4	DCI based TCI state switch delay	449

8.10A.5	RRC based TCI state switch delay	449

8.10A.6	Active TCI state list update delay	450

8.10B	Active TCI state switching delay for RedCap	450

8.10B.1	Introduction	450

8.10B.2	Known conditions for TCI state	450

8.10B.3	MAC-CE based TCI state switch delay	451

8.10B.4	DCI based TCI state switch delay	451

8.10B.5	RRC based TCI state switch delay	452

8.10B.6	Active TCI state list update delay	452

8.10C	Active TCI state switching delay for satellite access	453

8.10C.1	Introduction	453

8.10C.2	MAC-CE based TCI state switch delay	453

8.10C.4	DCI based TCI state switch delay	453

8.10C.5	RRC based TCI state switch delay	453

8.10C.6	Active TCI state list update delay	453

8.11	PSCell Change	454

8.11A	PSCell Change in Carriers with CCA	454

8.11B	Conditional PSCell Change	454

8.11B.1	Introduction	454

8.11B.2	Conditoinal PSCell Change delay	455

8.11B.2.1	Measurement time	455

8.11D	Conditional PSCell Change in Carriers with CCA	456

8.11D.1	Introduction	456

8.11D.2	Conditional PSCell Change delay	456

8.11D.2.1	Measurement time	457

8.12	Uplink spatial relation switch delay	457

8.12.1	Introduction	457

8.12.2	Known conditions for spatial relation when associated with DL-RS	457

8.12.3	MAC-CE based spatial relation switch delay	458

8.12.4	DCI based spatial relation switch delay	458

8.12.5	RRC based spatial relation switch delay	459

8.12A	Uplink spatial relation switch delay for RedCap	459

8.12A.1	Introduction	459

8.12A.2	Known conditions for spatial relation when associated with DL-RS	459

8.12A.3	MAC-CE based spatial relation switch delay	460

8.12A.4	DCI based spatial relation switch delay	460

8.12A.5	RRC based spatial relation switch delay	461

8.12C	Uplink spatial relation switch delay for satellite access	461

8.12C.1	Void	462

8.12C.2	Void	462

8.12C.3	Void	462

8.12C.4	Void	462

8.12C.5	Void	462

8.13	UE-specific CBW change	462

8.13.1	Introduction	462

8.13.2	UE-specific CBW change delay	462

8.13A	UE-specific CBW change for RedCap	462

8.13A.1	Introduction	462

8.13A.2	UE-specific CBW change delay	462

8.13C	UE-specific CBW change for satellite access	463

8.13C.1	Introduction	463

8.13C.2	UE-specific CBW change delay	463

8.14	Pathloss reference signal switching delay	463

8.14.1	Introduction	463

8.14.2	Known conditions for pathloss reference signal	464

8.14.3	MAC-CE based pathloss reference signal switch delay	464

8.14C	Pathloss reference signal switching delay for satellite access	464

8.14C.1	Introduction	465

8.14C.2	Known conditions for pathloss reference signal	465

8.14C.3	MAC-CE based pathloss reference signal switch delay	465

8.15	Active downlink TCI state switching delay for unified TCI	466

8.15.1	Introduction	466

8.15.4	DCI based downlink TCI state switch delay	467

8.15.5	Active Downlink TCI state list update delay	468

8.16	Active uplink TCI state switching delay for unified TCI	469

8.16.1	Introduction	469

8.16.4	DCI based uplink TCI state switch delay	471

8.16.5	Active Uplink TCI state list update delay	471

8.17	SCG Activation and Deactivation Delay	473

8.17.1	Introduction	473

8.17.2	SCG Activation Delay Requirement	473

8.17.3	SCG Deactivation Delay Requirement	474

8.18	TRP specific Link Recovery Procedures	474

8.18.1	Introduction	474

8.18.2	Requirements for TRP specific SSB based beam failure detection	475

8.18.2.1	Introduction	475

8.18.2.2	Minimum requirement	475

8.18.2.3	Measurement restriction for SSB based beam failure detection	477

8.18.3	Requirements for CSI-RS based beam failure detection	478

8.18.3.1	Introduction	478

8.18.3.2	Minimum requirement	478

8.18.3.3	Measurement restrictions for CSI-RS beam failure detection	480

8.18.4	Minimum requirement for L1 indication	481

8.18.5	Requirements for SSB based candidate beam detection	481

8.18.5.1	Introduction	481

8.18.5.2	Minimum requirement	481

8.18.5.3	Measurement restriction for SSB based candidate beam detection	483

8.18.6	Requirements for CSI-RS based candidate beam detection	484

8.18.6.1	Introduction	484

8.18.6.2	Minimum requirement	484

8.18.6.3	Measurement restriction for CSI-RS based candidate beam detection	486

8.18.7	Requirements for TRP specific Beam Failure Recovery	487

8.18.7.1	Introduction	487

8.18.7.2	Requirement	487

8.18.8	Scheduling availability of UE during TRP specific beam failure detection	488

8.18.8.1	Scheduling availability of UE performing TRP specific beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	488

8.18.8.2	Scheduling availability of UE performing TRP specific beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	488

8.18.8.3	Scheduling availability of UE performing TRP specific beam failure detection on FR2	488

8.18.8.4	Scheduling availability of UE performing TRP specific beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	489

8.18.9	Scheduling availability of UE during TRP specific candidate beam detection	489

8.18.9.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	489

8.18.9.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	489

8.18.9.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	489

8.18.9.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	490

8.19	Pre-configured measurement gap activation/deactivation delay	490

8.19.1	Introduction	490

8.19.2	Pre-configured measurement gap activation/deactivation upon DCI/timer-based BWP switch	490

8.19.2.1	Activation/deactivation upon DCI/timer-based BWP switch delay on a single CC	490

8.19.3	Pre-configured measurement gap activation/deactivation upon SCell activation/deactivation	491

8.19.4	Pre-configured measurement gap activation/deactivation upon RRC reconfiguration	491

9	Measurement Procedure	492

9.1	General measurement requirement	492

9.1.1	Introduction	492

9.1.2	Measurement gap	492

9.1.2.1	EN-DC: Measurement Gap Sharing	504

9.1.2.1a	SA: Measurement Gap Sharing	505

9.1.2.1b	NE-DC: Measurement Gap Sharing	506

9.1.2.1c	NR-DC: Measurement Gap Sharing	507

9.1.3	UE Measurement capability	508

9.1.3.1	EN-DC: Monitoring of multiple layers using gaps	508

9.1.3.1a	SA: Monitoring of multiple layers using gaps	508

9.1.3.1b	NE-DC: Monitoring of multiple layers using gaps	509

9.1.3.1c	NR-DC: Monitoring of multiple layers using gaps	509

9.1.3.2	EN-DC: Maximum allowed layers for multiple monitoring	510

9.1.3.2a	SA: Maximum allowed layers for multiple monitoring	511

9.1.3.2b	NE-DC: Maximum allowed layers for multiple monitoring	511

9.1.3.2c	NR-DC: Maximum allowed layers for multiple monitoring	512

9.1A.3.2	Void	513

9.1.3A	UE Measurement capability under operation mode with CCA	513

9.1.3A.1	EN-DC: Monitoring of multiple layers using gaps under CCA	513

9.1.3A.1a	SA: Monitoring of multiple layers using gaps under CCA	513

9.1.3A.2	EN-DC: Maximum allowed layers for multiple monitoring under CCA	513

9.1.3A.2a	SA: Maximum allowed layers for multiple monitoring under CCA	514

9.1.3C	UE Measurement capability under operation mode with satellite access	514

9.1.3C.1a	SA: Monitoring of multiple layers using gaps under satellite access	514

9.1.3C.2a	SA: Maximum allowed layers for multiple monitoring for SAN	515

9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria	515

9.1.4.1	Introduction	515

9.1.4.2	Requirements	515

9.1.5	Carrier-specific scaling factor	519

9.1.5.1	Monitoring of multiple layers outside gaps	520

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	522

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	523

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	524

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps	525

9.1.5.2	Monitoring of multiple layers within gaps	526

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	528

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	530

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps	532

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps	533

9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps	535

9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	536

9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	536

9.1.5.3	Monitoring of multiple layers within NCSG	536

9.1.5.3.1	SA mode: carrier-specific scaling factor for measurements performed within NCSG	536

9.1.6	Minimum requirement at transitions	537

9.1.7	Pre-configured measurement gap	538

9.1.7.1	Introduction	538

9.1.7.2	Requirements applicability	538

9.1.7.3	Requirements	538

9.1.7.3.1	Requirements for autonomous activation/deactivation mechanism	539

9.1.7.3.2	Requirements for network-controlled activation/deactivation mechanism	539

9.1.7.3.3	Requirements for reception/transmission during activation/deactivation	540

9.1.8	Concurrent measurement gaps	540

9.1.8.1	Introduction	540

9.1.8.2	Requirements	540

9.1.8.3	Collision between concurrent measurement gaps	542

9.1.8.4	Measurement gap related requirements of concurrent measurement gaps	542

9.1.9	Network controlled small gap	542

9.1.9.1	Introduction	542

9.1.9.2	Requirements applicability	543

9.1.10	MUSIM gaps	545

9.1.11	UL gap for Tx power management	546

9.1A	General measurement requirement for RedCap	547

9.1A.1	Introduction	547

9.1A.2	Measurement gap	547

9.1A.2.1	SA: Measurement Gap Sharing	551

9.1A.3	UE Measurement capability	552

9.1A.3.1	SA: Monitoring of multiple layers using gaps	552

9.1A.3.2	SA: Maximum allowed layers for multiple monitoring	552

9.1A.4	Capabilities for Support of Event Triggering and Reporting Criteria	552

9.1A.4.1	Introduction	552

9.1A.4.2	Requirements	553

9.1A.5	Carrier-specific scaling factor	553

9.1A.5.1	Monitoring of multiple layers outside gaps	553

9.1A.5.1.1	SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps	554

9.1A.5.2	Monitoring of multiple layers within gaps	554

9.1A.5.2.1	SA mode: carrier-specific scaling factor for SSB measurements performed within gaps	554

9.1A.6	Minimum requirement at transitions	555

9.1C	General measurement requirement	556

9.1C.1	Introduction	556

9.1C.2	Measurement gap	556

9.1C.8	Concurrent measurement gaps for SAN	558

9.1C.8.1	Introduction	558

9.1C.8.2	Requirements	558

9.1C.8.3	Collision between concurrent measurement gaps	559

9.1C.8.4	Measurement gap related requirements of concurrent measurement gaps	559

9.1C.9	Collision between SMTC and measurement gap for SAN	559

9.1C.9.1	Introduction	559

9.1C.9.2	Collision between SMTCs and measurement gap	559

9.1C.9.3	Collision between multiple SMTCs on a SAN carrier	560

9.2	NR intra-frequency measurements	560

9.2.1	Introduction	560

9.2.2	Requirements applicability	561

9.2.3	Number of cells and number of SSB	562

9.2.3.1	Requirements for FR1	562

9.2.3.2	Requirements for FR2	562

9.2.4	Measurement Reporting Requirements	562

9.2.4.1	Periodic Reporting	562

9.2.4.2	Event-triggered Periodic Reporting	562

9.2.4.3	Event Triggered Reporting	562

9.2.5	Intrafrequency measurements without measurement gaps	563

9.2.5.1	Intrafrequency cell identification	563

9.2.5.2	Measurement period	568

9.2.5.3	Scheduling availability of UE during intra-frequency measurements	571

9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	571

9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	571

9.2.5.3.3	Scheduling availability of UE performing measurements on FR2	572

9.2.5.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	573

9.2.5.4	SFTD Measurements between PCell and PSCell	574

9.2.5.4.1	Introduction	574

9.2.5.4.2	SFTD Measurement delay	574

9.2.5.4.3	SFTD Measurement Reporting Delay	574

9.2.6	Intra-frequency measurements with measurement gaps	575

9.2.6.1	Void	575

9.2.6.2	Intra-frequency cell identification	575

9.2.6.3	Intrafrequency Measurement Period	577

9.2.7	Intra-frequency measurements with NCSG	579

9.2.7.1	Intra-frequency cell identification	579

9.2.7.2	Measurement period	581

9.2.7.3	Scheduling availability during intra-frequency measurement with NCSG	582

9.2A	NR intra-frequency measurements with CCA	582

9.2A.1	Introduction	582

9.2A.2	Requirements applicability	583

9.2A.3	Number of cells and number of SSB	583

9.2A.3.1	Requirements for FR1	583

9.2A.3.2	Requirements for FR2-2	584

9.2A.4	Measurement Reporting Requirements	584

9.2A.5	Intra-frequency measurements without measurement gaps	585

9.2A.5.2	Measurement period	589

9.2A.5.3	Scheduling availability of UE during intra-frequency measurements	591

9.2A.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	592

9.2A.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	592

9.2A.5.3.3	Scheduling availability of UE performing measurements in TDD bands on FR2-2	592

9.2A.6	Intra-frequency measurements with measurement gaps	592

9.2A.6.1	Intra-frequency cell identification	592

9.2A.6.2	Intra-frequency Measurement Period	595

9.2A.7	Intra-frequency RSSI and Channel occupancy measurements	596

9.2A.7.1	Intra-frequency RSSI measurements	596

9.2A.7.2	Intra-frequency Channel occupancy measurements	598

9.2A.7.3	Scheduling restriction during RSSI and Channel Occupancy measurements in FR1	599

9.2A.7.4	Scheduling restriction during RSSI measurements in FR2-2	599

9.2B	NR intra-frequency measurements for RedCap	600

9.2B.1	Introduction	600

9.2B.2	Requirements applicability	600

9.2B.3	Number of cells and number of SSB	601

9.2B.3.1	Requirements for FR1	601

9.2B.3.2	Requirements for FR2	601

9.2B.4	Measurement Reporting Requirements	601

9.2B.4.1	Periodic Reporting	601

9.2B.4.2	Event-triggered Periodic Reporting	601

9.2B.4.3	Event Triggered Reporting	601

9.2B.5	Intra-frequency measurements without measurement gaps for RedCap	602

9.2B.5.1	Intra-frequency cell identification	602

9.2B.5.2	Measurement period	604

9.2B.5.3	Scheduling availability of UE during intra-frequency measurements	605

9.2B.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	605

9.2B.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	606

9.2B.5.3.3	Scheduling availability of UE performing measurements on FR2	606

9.2B.5.3.4	Scheduling availability of UE performing measurements in HD-FDD bands on FR1	607

9.2B.6	Intra-frequency measurements with measurement gaps	607

9.2B.6.1	Intra-frequency cell identification	607

9.2B.6.2	Intra-frequency Measurement Period	609

9.2C	NR intra-frequency measurements for SAN	609

9.2C.1	Introduction	609

9.2C.2	Requirements applicability	610

9.2C.3	Number of cells and number of SSB	610

9.2C.3.1	Requirements for FR1	610

9.2C.4	Measurement Reporting Requirements	611

9.2C.4.1	Periodic Reporting	611

9.2C.4.2	Event-triggered Periodic Reporting	611

9.2C.4.3	Event Triggered Reporting	611

9.2C.5	Intra frequency measurements without measurement gaps	611

9.2C.5.1	Intra frequency cell identification	611

9.2C.5.2	Measurement period	614

9.2C.5.3	Scheduling availability of UE during intra-frequency measurements	614

9.2C.5.3.1	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	614

9.2C.5.3.2	Scheduling availability of UE performing measurements on a neighbor cell served by a different satellite in LEO	614

9.2C.6	Intra-frequency measurements with measurement gaps	615

9.2C.6.1	Intra-frequency cell identification	615

9.2C.6.3	Intrafrequency Measurement Period	616

9.3	NR inter-frequency measurements	617

9.3.1	Introduction	617

9.3.2	Requirements applicability	618

9.3.2.1	Void	618

9.3.2.2	Void	618

9.3.3	Number of cells and number of SSB	618

9.3.3.1	Requirements for FR1	618

9.3.3.2	Requirements for FR2	618

9.3.4	Inter-frequency measurement with measurement gaps	618

9.3.4.1	Void	622

9.3.4.2	Void	622

9.3.5	Inter-frequency measurements	622

9.3.5.1	Void	623

9.3.5.2	Void	623

9.3.5.3	Void	623

9.3.6	Inter-frequency measurements reporting requirements	623

9.3.6.1	Periodic Reporting	623

9.3.6.2	Event-triggered Periodic Reporting	623

9.3.6.3	Event-triggered Reporting	623

9.3.7	Void	624

9.3.8	Inter-frequency SFTD measurement requirements	624

9.3.8.1	Introduction	624

9.3.8.2	SFTD Measurement delay	624

9.3.8.3	SFTD Measurement reporting delay	625

9.3.9	Inter frequency measurements without measurement gaps	625

9.3.9.1	Inter frequency Cell identification	625

9.3.9.2	Measurement period	628

9.3.9.3	Scheduling availability of UE during inter-frequency measurements	629

9.3.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	630

9.3.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	630

9.3.9.3.3	Scheduling availability of UE performing measurements on FR2	631

9.3.9.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	631

9.3.10	Inter-frequency measurement with NCSG	631

9.3.10.1	Inter-frequency cell identification	631

9.3.10.2	Measurement period	633

9.3.10.3	Scheduling availability during inter-frequency measurement with NCSG	633

9.3.10.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	633

9.3.10.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	634

9.3.10.3.3	Scheduling availability of UE performing measurements on FR2	635

9.3.10.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	637

9.3A	NR inter-frequency measurements in carrier frequencies with CCA	637

9.3A.1	Introduction	637

9.3A.2	Requirements applicability	638

9.3A.3	Number of cells and number of SSB	638

9.3A.3.1	Requirements for FR1	638

9.3A.3.2	Requirements for FR2-2	638

9.3A.4	Inter-frequency cell identification	638

9.3A.5	Inter-frequency measurements	641

9.3A.6	NR Inter-frequency measurements reporting requirements	642

9.3A.6.1	Periodic Reporting	642

9.3A.6.2	Event-triggered Periodic Reporting	642

9.3A.6.3	Event-triggered Reporting	642

9.3A.8	Inter-frequency RSSI measurements	643

9.3A.9	Inter-frequency channel occupancy measurements	644

9.3B	NR inter-frequency measurements for RedCap	644

9.3B.1	Introduction	644

9.3B.2	Requirements applicability	645

9.3B.3	Number of cells and number of SSB	645

9.3B.3.1	Requirements for FR1	645

9.3B.3.2	Requirements for FR2	645

9.3B.4	Inter-frequency measurement with measurement gaps	646

9.3B.5	Inter-frequency measurements	647

9.3B.6	Inter-frequency measurements reporting requirements	648

9.3B.6.1	Periodic Reporting	648

9.3B.6.2	Event-triggered Periodic Reporting	648

9.3B.6.3	Event-triggered Reporting	648

9.3B.7	Inter frequency measurements without measurement gaps	649

9.3B.7.1	Inter frequency Cell identification	649

9.3B.7.2	Measurement period	651

9.3B.7.3	Scheduling availability of UE during inter-frequency measurements	652

9.3B.7.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	652

9.3B.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	653

9.3B.7.3.3	Scheduling availability of UE performing measurements on FR2	653

9.3B.7.3.4	Scheduling availability of UE performing measurements in HD-FDD bands on FR1	653

9.3C	NR inter-frequency measurements for SAN	654

9.3C.1	Introduction	654

9.3C.2	Requirements applicability	654

9.3C.3	Number of cells and number of SSB	654

9.3C.3.1	Requirements for FR1	654

9.3C.4	Inter-frequency measurement with measurement gaps	655

9.3C.5	Inter-frequency measurements	656

9.3C.6	Inter-frequency measurements reporting requirements	656

9.3C.6.1	Periodic Reporting	656

9.3C.6.2	Event-triggered Periodic Reporting	656

9.3C.6.3	Event-triggered Reporting	657

9.3C.7	Inter frequency measurements without measurement gaps	657

9.3C.7.1	Inter frequency Cell identification	657

9.3C.7.2	Measurement period	659

9.3C.7.3	Scheduling availability of UE during inter-frequency measurements	659

9.3C.7.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	659

9.3C.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	660

9.4	Inter-RAT measurements	660

9.4.1	Introduction	660

9.4.2	NR − E-UTRAN FDD measurements	662

9.4.2.1	Introduction	662

9.4.2.2	Requirements when no DRX is used	662

9.4.2.3	Requirements when DRX is used	663

9.4.2.4	Measurement reporting requirements	665

9.4.2.4.1	Periodic Reporting	665

9.4.2.4.2	Event-Triggered Periodic Reporting	665

9.4.2.4.3	Event-Triggered Reporting	666

9.4.3	NR − E-UTRAN TDD measurements	666

9.4.3.1	Introduction	666

9.4.3.2	Requirements when no DRX is used	666

9.4.3.3	Requirements when DRX is used	668

9.4.3.4	Measurement reporting requirements	669

9.4.3.4.1	Periodic Reporting	669

9.4.3.4.2	Event-Triggered Periodic Reporting	670

9.4.3.4.3	Event-Triggered Reporting	670

9.4.3.5	Scheduling Availability During NR − E-UTRAN TDD measurements with NCSG	670

9.4.4	Inter-RAT RSTD measurements	670

9.4.4.1	NR − E-UTRAN FDD RSTD measurements	670

9.4.4.1.1	Introduction	670

9.4.4.1.2	Requirements	671

9.4.4.2	NR − E-UTRAN TDD RSTD measurements	674

9.4.4.2.1	Introduction	674

9.4.4.2.2	Requirements	675

9.4.5	Inter-RAT E-CID measurements	678

9.4.5.1	NR−E-UTRAN FDD E-CID RSRP and RSRQ measurements	678

9.4.5.1.1	Introduction	678

9.4.5.1.2	Requirements	678

9.4.5.1.3	Measurement Reporting Delay	678

9.4.5.2	NR−E-UTRAN TDD E-CID RSRP and RSRQ measurements	678

9.4.5.2.1	Introduction	678

9.4.5.2.2	Requirements	678

9.4.5.2.3	Measurement Reporting Delay	679

9.4.6	NR − UTRAN FDD measurements	679

9.4.6.1	Introduction	679

9.4.6.2	Requirements when no DRX is used	679

9.4.6.3	Requirements when DRX is used	680

9.4.7	NR – E-UTRAN measurements with autonomous gaps	682

9.4.7.1	CGI identification of an E-UTRA cell with autonomous gaps	682

9.4A	Inter-RAT measurements for RedCap	683

9.4A.1	Introduction	683

9.4A.2	NR − E-UTRAN FDD measurements	684

9.4A.2.1	Introduction	684

9.4A.2.2	Requirements when no DRX is used	684

9.4A.2.3	Requirements when DRX is used	685

9.4A.2.4	Measurement reporting requirements	686

9.4A.2.4.1	Periodic Reporting	686

9.4A.2.4.2	Event-Triggered Periodic Reporting	686

9.4A.2.4.3	Event-Triggered Reporting	686

9.4A.3	NR − E-UTRAN TDD measurements	687

9.4A.3.1	Introduction	687

9.4A.3.2	Requirements when no DRX is used	687

9.4A.3.3	Requirements when DRX is used	689

9.4A.3.4	Measurement reporting requirements	690

9.4A.3.4.1	Periodic Reporting	690

9.4A.3.4.2	Event-Triggered Periodic Reporting	690

9.4A.3.4.3	Event-Triggered Reporting	691

9.4A.4	NR – E-UTRAN measurements with autonomous gaps	691

9.4A.4.1	CGI identification of an E-UTRA cell with autonomous gaps	691

9.4A.4.2	CGI reporting delay	692

9.4A.4.3	CGI reporting scheduling restriction	692

9.5	L1-RSRP measurements for Reporting	692

9.5.1	Introduction	692

9.5.2	Requirements applicability	693

9.5.3	Measurement Reporting Requirements	693

9.5.3.1	Periodic Reporting	693

9.5.3.2	Semi-Persistent Reporting	694

9.5.3.3	Aperiodic Reporting	694

9.5.4	L1-RSRP measurement requirements	694

9.5.4.1	SSB based L1-RSRP Reporting	694

9.5.4.2	CSI-RS based L1-RSRP Reporting	697

9.5.4A	Void	701

9.5.4A.1	Void	701

9.5.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	701

9.5.5.1	Measurement restriction for SSB based L1-RSRP	701

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP	701

9.5.6	Scheduling availability of UE during L1-RSRP measurement	702

9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	702

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	702

9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	703

9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	704

9.5A	L1-RSRP measurements for Reporting under CCA	704

9.5A.1	Introduction	704

9.5A.2	Requirements applicability	705

9.5A.3	Measurement Reporting Requirements	705

9.5A.3.1	Periodic Reporting	705

9.5A.3.2	Semi-Persistent Reporting	705

9.5A.3.3	Aperiodic Reporting	706

9.5A.4	L1-RSRP measurement requirements	706

9.5A.4.1	SSB based L1-RSRP Reporting	706

9.5A.5	Measurement restriction for L1-RSRP measurement	709

9.5A.5.1	Measurement restriction for SSB based L1-RSRP	709

9.5A.6	Scheduling availability of UE during L1-RSRP measurement	709

9.5A.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	709

9.5A.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	709

9.5A.6.3	Scheduling availability of UE performing L1-RSRP measurement in case of FR1-FR2 inter-band CA	709

9.5A.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2-2	709

9.5A.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	710

9.5B	L1-RSRP measurements for Reporting for RedCap	710

9.5B.1	Introduction	710

9.5B.2	Requirements applicability	711

9.5B.3	Measurement Reporting Requirements	711

9.5B.3.1	Periodic Reporting	711

9.5B.3.2	Semi-Persistent Reporting	712

9.5B.3.3	Aperiodic Reporting	712

9.5B.4	L1-RSRP measurement requirements	712

9.5B.4.1	SSB based L1-RSRP Reporting	712

9.5B.4.2	CSI-RS based L1-RSRP Reporting	714

9.5B.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	717

9.5B.5.1	Measurement restriction for SSB based L1-RSRP	717

9.5B.5.2	Measurement restriction for CSI-RS based L1-RSRP	718

9.5B.6	Scheduling availability of UE during L1-RSRP measurement	718

9.5B.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	718

9.5B.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	718

9.5B.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	719

9.5C	L1-RSRP measurements for Reporting for satellite access	719

9.5C.1	Introduction	719

9.5C.3	Measurement Reporting Requirements	720

9.5C.3.1	Periodic Reporting	720

9.5C.3.2	Semi-Persistent Reporting	720

9.5C.3.3	Aperiodic Reporting	720

9.5C.4	L1-RSRP measurement requirements	721

9.5C.4.1	SSB based L1-RSRP Reporting	721

9.5C.5	Measurement restriction for L1-RSRP measurement	722

9.5C.5.1	Measurement restriction for SSB based L1-RSRP	723

9.5C.5.2	Measurement restriction for CSI-RS based L1-RSRP	723

9.5C.6	Scheduling availability of UE during L1-RSRP measurement	723

9.5C.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	723

9.5C.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	723

9.6	NE-DC: Measurements	724

9.6.1	Introduction	724

9.6.2	SFTD Measurements	724

9.6.2.1	Introduction	724

9.6.2.2	SFTD Measurement requirements	724

9.7	Cross Link Interference measurements	725

9.7.1	Introduction	725

9.7.2	SRS-RSRP measurements	725

9.7.2.1	Introduction	725

9.7.2.2	Requirements applicability	725

9.7.2.3	Measurement Reporting Requirements	725

9.7.2.3.1	Periodic Reporting	726

9.7.2.3.2	Event-triggered Periodic Reporting	726

9.7.2.3.3	Event Triggered Reporting	726

9.7.2.4	Measurement capability	726

9.7.2.5	SRS-RSRP measurement period	726

9.7.3	CLI-RSSI measurements	727

9.7.3.1	Introduction	727

9.7.3.2	Requirements applicability	727

9.7.3.3	Measurement Reporting Requirements	727

9.7.3.3.1	Periodic Reporting	727

9.7.3.3.2	Event-triggered Periodic Reporting	727

9.7.3.3.3	Event Triggered Reporting	727

9.7.3.4	Measurement capability	727

9.7.3.5	CLI-RSSI measurement period	728

9.7.4	Scheduling availability of UE during CLI measurements	728

9.7.4.1	Scheduling availability of UE performing measurement on FR1	728

9.7.4.2	Scheduling availability of UE performing measurement on FR2	728

9.8	L1-SINR measurements for Reporting	729

9.8.1	Introduction	729

9.8.2	Requirements applicability	729

9.8.3	Measurement Reporting Requirements	730

9.8.3.1	Periodic Reporting	730

9.8.3.2	Semi-Persistent Reporting	730

9.8.4	L1-SINR measurement requirements	731

9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	731

9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	735

9.8.5	Measurement restriction for L1-SINR measurement	737

9.8.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	737

9.8.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	738

9.8.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	739

9.8.6	Scheduling availability of UE during L1-SINR measurement	739

9.8.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	739

9.8.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	739

9.8.6.4	Scheduling availability of UE performing L1-SINR measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	740

9.9	 NR measurements for positioning	740

9.9.1	Introduction	740

9.9.1.1	General Aspects of Gap-based Measurement	741

9.9.1.2	General Aspects of Gapless Measurement	741

9.9.1.3	Scheduling Availability of UE during PRS Measurement without Measurement Gaps	742

9.9.2	RSTD measurements	743

9.9.2.1	Introduction	743

9.9.2.2	Requirements Applicability	743

9.9.2.3	Measurement Capability	743

9.9.2.4	Measurement Reporting Requirements	743

9.9.2.4.1	Void	744

9.9.2.4.2	Void	744

9.9.2.4.3	Void	744

9.9.2.5	Measurements Period Requirements	744

9.9.2.6	Void	747

9.9.2.7	Measurements Period Requirements without Measurement Gaps	747

9.9.2.8	Void	750

9.9.2.9	Measurements Period Requirements with both MG and PPW	750

9.9.3	PRS-RSRP measurements	750

9.9.3.1	Introduction	750

9.9.3.2	Requirements applicability	750

9.9.3.3	Measurement Capability	750

9.9.3.4	Measurement Reporting Requirements	750

9.9.3.5	Measurement Period Requirements	751

9.9.3.6	Measurement Period Requirements without Measurement Gaps	753

9.9.3.7	Void	756

9.9.3.8	Measurements Period Requirements with both MG and PPW	756

9.9.4	UE Rx-Tx time difference measurements	756

9.9.4.1 Introduction	756

9.9.4.2 Requirements Applicability	756

9.9.4.3 Measurement Capability	757

9.9.4.4 Measurement Reporting Requirements	757

9.9.4.5	Measurement Period Requirements	757

9.9.4.6	Measurement Period Requirements without Measurement Gaps	760

9.9.4.7	Void	764

9.9.4.8	Measurements Period Requirements with both MG and PPW	764

9.9.5	E-CID measurements	764

9.9.5.1	Introduction	764

9.9.5.2	Measurement Requirements	765

9.9.5.2.1	Intra-frequency Measurement Requirements	765

9.9.5.2.2	Inter-frequency Measurement Requirements	765

9.9.5.2.3	Measurement Reporting Delay	765

9.9.6	PRS-RSRPP measurements	766

9.9.6.1	Introduction	766

9.9.6.2	Requirements applicability	766

9.9.6.3	Measurement capability	766

9.9.6.4	Measurement reporting requirements	766

9.9.6.5	Measurement period requirements	766

9.9.6.6	Measurement Period Requirements without Measurement Gaps	766

9.9.6.7	Void	766

9.9.6.8	Measurements Period Requirements with both MG and PPW	766

9.10	CSI-RS based L3 measurements	767

9.10.1	Introduction	767

9.10.2	CSI-RS based intra-frequency measurements	767

9.10.2.1	Introduction	767

9.10.2.2	Requirements applicability	767

9.10.2.3	Number of cells and number of CSI-RS	768

9.10.2.3.1	Requirements for FR1	768

9.10.2.3.2	Requirements for FR2	769

9.10.2.4	Measurement Reporting Requirements	769

9.10.2.4.1	Periodic Reporting	769

9.10.2.4.2	Event-triggered Periodic Reporting	769

9.10.2.4.3	Event Triggered Reporting	769

9.10.2.5	Intra-frequency measurements without measurement gaps	770

9.10.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	771

9.10.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	771

9.10.2.6.2	Scheduling availability of UE performing CSI-RS based measurements in FR2	772

9.10.3	CSI-RS based Inter-frequency measurements	772

9.10.3.1	Introduction	772

9.10.3.2	Requirements applicability	772

9.10.3.3	Number of cells and number of CSI-RS resources	773

9.10.3.3.1	Requirements for FR1	773

9.10.3.3.2	Requirements for FR2	773

9.10.3.4	Measurements reporting requirements	773

9.10.3.4.1	Periodic Reporting	773

9.10.3.4.2	Event-triggered Periodic Reporting	773

9.10.3.4.3	Event-triggered Reporting	774

9.10.3.5	Inter frequency measurements with measurement gaps	774

9.11	NR measurements with autonomous gaps	775

9.11.1	Introduction	775

9.11.2	CGI identification of an NR cell with autonomous gaps	776

9.11.3	CGI reporting delay	777

9.11A	NR measurements with autonomous gaps for RedCap	777

9.11A.1	Introduction	777

9.11A.2	CGI identification of an NR cell with autonomous gaps	777

9.11A.3	CGI reporting delay	778

9.11A.4	CGI reporting scheduling restriction	778

9.12	Measurement for Propagation Delay Compensation	779

9.12.1	Introduction	779

9.12.2	Requirements Applicability	779

9.12.3	Measurement Capability	779

9.12.4	Measurement period requirements	779

9.12.4.1	PRS Measurement Period	779

9.12.4.2	TRS Measurement Period	780

9.12.5	Measurement Reporting Requirements	781

9.12.6	Scheduling availability during measurement for Propagation Delay Compensation	781

9.12.7	Measurement restriction for measurement for Propagation Delay Compensation	782

9.13	L1-RSRP measurements for a cell with different PCI from serving cell	782

9.13.1	Introduction	782

9.13.2	Requirements Applicability	782

9.13.3	Measurement Reporting Requirements	783

9.13.3.1	Periodic Reporting	783

9.13.3.2	Semi-Persistent Reporting	783

9.13.3.3	Aperiodic Reporting	783

9.13.4	L1-RSRP measurement requirements	784

9.13.4.1	Inter-cell SSB based L1-RSRP Reporting	784

9.13.5	Measurement restriction for L1-RSRP measurement	785

9.13.5.1	Measurement restriction for SSB based L1-RSRP	785

9.13.6	Scheduling availability of UE during L1-RSRP measurement	786

9.13.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	786

9.13.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	786

9.13.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	787

9.13.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	787

9.13.6.5	Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1	787

10	Measurement Performance requirements	783

10.1	NR measurements	783

10.1.1	Introduction	783

10.1.2	Intra-frequency RSRP accuracy requirements for FR1	783

10.1.2.1	Intra-frequency SS-RSRP accuracy requirements	783

10.1.2.1.1	Absolute SS-RSRP Accuracy	783

10.1.2.1.2	Relative SS-RSRP Accuracy	784

10.1.2.2	Void	785

10.1.2.3	Intra-frequency CSI-RSRP accuracy requirements	785

10.1.2.3.1	Absolute CSI-RSRP Accuracy	785

10.1.2.3.2	Relative CSI-RSRP Accuracy	786

10.1.2B	Intra-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	787

10.1.2B.1	Intra-frequency SS-RSRP accuracy requirements	787

10.1.2B.1.1	Absolute SS-RSRP Accuracy	787

10.1.2C	Intra-frequency RSRP accuracy requirements for FR1 SAN	788

10.1.2C.1	Intra-frequency SS-RSRP accuracy requirements	788

10.1.2C.1.1	Absolute SS-RSRP Accuracy	788

10.1.2C.1.2	Relative SS-RSRP Accuracy	789

10.1.3	Intra-frequency RSRP accuracy requirements for FR2	789

10.1.3.1	Intra-frequency SS-RSRP accuracy requirements	789

10.1.3.1.1	Absolute SS-RSRP Accuracy	789

10.1.3.1.2	Relative SS-RSRP Accuracy	790

10.1.3.2	Void	791

10.1.3.3	Intra-frequency CSI-RSRP accuracy requirements	791

10.1.3.3.1	Absolute CSI-RSRP Accuracy	791

10.1.3.3.2	Relative CSI-RSRP Accuracy	792

10.1.3B	Intra-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	793

10.1.3B.1	Intra-frequency SS-RSRP accuracy requirements	793

10.1.3B.1.1	Absolute SS-RSRP Accuracy	793

10.1.4	Inter-frequency RSRP accuracy requirements for FR1	794

10.1.4.1	Inter-frequency SS-RSRP accuracy requirements	794

10.1.4.1.1	Absolute Accuracy of SS-RSRP in FR1	794

10.1.4.1.2	Relative Accuracy of SS-RSRP in FR1	795

10.1.4.2	Void	796

10.1.4.3	Inter-frequency CSI-RSRP accuracy requirements	796

10.1.4.3.1	Absolute Accuracy of CSI-RSRP in FR1	796

10.1.4.3.2	Relative Accuracy of CS-RSRP in FR1	797

10.1.4B	Inter-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	798

10.1.4B.1	Inter-frequency SS-RSRP accuracy requirements	798

10.1.4B.1.1	Absolute Accuracy of SS-RSRP in FR1	798

10.1.4C	Inter-frequency RSRP accuracy requirements for FR1 SAN	799

10.1.4C.1	Inter-frequency SS-RSRP accuracy requirements	799

10.1.4C.1.1	Absolute Accuracy of SS-RSRP in FR1	799

10.1.4C.1.2	Relative Accuracy of SS-RSRP in FR1	800

10.1.5	Inter-frequency RSRP accuracy requirements for FR2	800

10.1.5.1	Inter-frequency SS-RSRP accuracy requirements	800

10.1.5.1.1	Absolute SS-RSRP Accuracy	800

10.1.5.1.2	Relative SS-RSRP Accuracy	801

10.1.5.2	Void	802

10.1.5.3	Inter-frequency CSI-RSRP accuracy requirements	802

10.1.5.3.1	Absolute CSI-RSRP Accuracy	802

10.1.5.3.2	Relative CSI-RSRP Accuracy	803

10.1.5B	Inter-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	804

10.1.5B.1	Inter-frequency SS-RSRP accuracy requirements	804

10.1.5B.1.1	Absolute SS-RSRP Accuracy	804

10.1.6	RSRP Measurement Report Mapping	805

10.1.7	Intra-frequency RSRQ accuracy requirements for FR1	807

10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	807

10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1	807

10.1.7.2	Intra-frequency CSI-RSRQ accuracy requirements	808

10.1.7.2.1	Absolute CSI-RSRQ Accuracy	808

10.1.7B	Intra-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	809

10.1.7B.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	809

10.1.7B.1.1	Absolute SS-RSRQ Accuracy in FR1	809

10.1.7C	Intra-frequency RSRQ accuracy requirements for FR1 SAN	810

10.1.7C.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	810

10.1.7C.1.1	Absolute SS-RSRQ Accuracy in FR1	810

10.1.8	Intra-frequency RSRQ accuracy requirements for FR2	811

10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	811

10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2	811

10.1.8.2	Intra-frequency CSI-RSRQ accuracy requirements	812

10.1.8.2.1	Absolute CSI-RSRQ Accuracy	812

10.1.8B	Intra-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	812

10.1.8B.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	812

10.1.8B.1.1	Absolute SS-RSRQ Accuracy in FR2	813

10.1.9	Inter-frequency RSRQ accuracy requirements for FR1	813

10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	813

10.1.9.1.1	Absolute Accuracy of SS-RSRQ in FR1	813

10.1.9.1.2	Relative Accuracy of SS-RSRQ in FR1	814

10.1.9.2	Inter-frequency CSI-RSRQ accuracy requirements	815

10.1.9.2.1	Absolute CSI-RSRQ Accuracy	815

10.1.9.2.2	Relative CSI-RSRQ Accuracy	816

10.1.9B	Inter-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	817

10.1.9B.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	817

10.1.9B.1.1	Absolute Accuracy of SS-RSRQ in FR1	817

10.1.9C	Inter-frequency RSRQ accuracy requirements for FR1 SAN	818

10.1.9C.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	818

10.1.9C.1.1	Absolute Accuracy of SS-RSRQ in FR1	818

10.1.9C.1.2	Relative Accuracy of SS-RSRQ in FR1	819

10.1.10	Inter-frequency RSRQ accuracy requirements for FR2	820

10.1.10.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	820

10.1.10.1.1	Absolute Accuracy of SS-RSRQ in FR2	820

10.1.10.1.2	Relative Accuracy of SS-RSRQ in FR2	820

10.1.10.2	Inter-frequency CSI-RSRQ accuracy requirements	821

10.1.10.2.1	Absolute CSI-RSRQ Accuracy	821

10.1.10.2.2	Relative CSI-RSRQ Accuracy	822

10.1.10B	 Inter-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	823

10.1.10B.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	823

10.1.10B.1.1	Absolute Accuracy of SS-RSRQ in FR2	823

10.1.11	RSRQ report mapping	824

10.1.12	Intra-frequency SINR accuracy requirements for FR1	824

10.1.12.1	Intra-frequency SS-SINR accuracy requirements in FR1	824

10.1.12.1.1	Absolute SS-SINR Accuracy in FR1	824

10.1.12.2	Intra-frequency CSI-SINR accuracy requirements in FR1	825

10.1.12.2.1	Absolute CSI-SINR Accuracy in FR1	825

10.1.12C	Intra-frequency SINR accuracy requirements for FR1 SAN	826

10.1.12C.1	Intra-frequency SS-SINR accuracy requirements in FR1	826

10.1.12C.1.1	Absolute SS-SINR Accuracy in FR1	826

10.1.13	Intra-frequency SINR accuracy requirements for FR2	827

10.1.13.1	Intra-frequency SS-SINR accuracy requirements in FR2	827

10.1.13.1.1	Absolute SS-SINR Accuracy in FR2	827

10.1.13.2	Intra-frequency CSI-SINR accuracy requirements in FR2	828

10.1.13.2.1	Absolute CSI-SINR Accuracy in FR2	828

10.1.14	Inter-frequency SINR accuracy requirements for FR1	828

10.1.14.1	Inter-frequency SS-SINR accuracy requirements in FR1	828

10.1.14.1.1	Aboslute Accuracy of SS-SINR in FR1	828

10.1.14.1.2	Relative Accuracy of SS-SINR in FR1	829

10.1.14.2	Inter-frequency CSI-SINR accuracy requirements in FR1	830

10.1.14.2.1	Aboslute Accuracy of CSI-SINR in FR1	830

10.1.14.2.2	Relative Accuracy of CSI-SINR in FR1	831

10.1.14C	Inter-frequency SINR accuracy requirements for FR1 SAN	832

10.1.14C.1	Inter-frequency SS-SINR accuracy requirements in FR1	832

10.1.14C.1.1	Aboslute Accuracy of SS-SINR in FR1	832

10.1.14C.1.2	Relative Accuracy of SS-SINR in FR1	833

10.1.15	Inter-frequency SINR accuracy requirements for FR2	834

10.1.15.1	Inter-frequency SS-SINR accuracy requirements in FR2	834

10.1.15.1.1	Aboslute Accuracy of SS-SINR in FR2	834

10.1.15.1.2	Relative Accuracy of SS-SINR in FR2	834

10.1.15.2	Inter-frequency CSI-SINR accuracy requirements in FR2	835

10.1.15.2.1	Aboslute Accuracy of CSI-SINR in FR2	835

10.1.15.2.2	Relative Accuracy of CSI-SINR in FR2	836

10.1.16	SINR report mapping	837

10.1.16.1	SS-SINR and CSI-SINR measurement report mapping	837

10.1.17	Power Headroom	838

10.1.17.1	Power Headroom Report	838

10.1.17.1.1	Power Headroom Report Mapping	838

10.1.18	PCMAX,c,f	839

10.1.18.1	Report Mapping	839

10.1.19	L1-RSRP accuracy requirements for FR1	839

10.1.19.1	SSB based L1-RSRP accuracy requirements	839

10.1.19.1.1	Absolute Accuracy	839

10.1.19.1.2	Relative Accuracy	840

10.1.19.2	CSI-RS based L1-RSRP accuracy requirements	841

10.1.19.2.1	Absolute Accuracy	841

10.1.19.2.2	Relative Accuracy	842

10.1.19C	L1-RSRP accuracy requirements for FR1 SAN	843

10.1.19C.1	SSB based L1-RSRP accuracy requirements	843

10.1.19C.1.1	Absolute Accuracy	843

10.1.19C.1.2	Relative Accuracy	844

10.1.20	L1-RSRP accuracy requirements for FR2	844

10.1.20.1	SSB based L1-RSRP accuracy requirements	844

10.1.20.1.1	Absolute Accuracy	844

10.1.20.1.2	Relative Accuracy	845

10.1.20.2	CSI-RS based L1-RSRP accuracy requirements	846

10.1.20.2.1	Absolute Accuracy	846

10.1.20.2.2	Relative Accuracy	847

10.1.21	SFTD accuracy requirements	848

10.1.21.1	SFTD acuracy requirements for NE-DC	848

10.1.21.2	SFTD acuracy requirements for NR-DC	850

10.1.21.3	Inter frequency SFTD acuracy requirements	851

10.1.22	CLI measurement accuracy requirements	852

10.1.22.1	SRS-RSRP	852

10.1.22.1.1	SRS-RSRP Accuracy	852

10.1.22.1.2	SRS-RSRP report mapping	854

10.1.22.2	CLI-RSSI	854

10.1.22.2.1	CLI-RSSI Accuracy	854

10.1.22.2.2	CLI-RSSI report mapping	855

10.1.23	RSTD Measurements	856

10.1.23.1	Introduction	856

10.1.23.2	Measurement Accuracy Requirements	856

10.1.23.3	Report mapping	863

10.1.23.3.1	Absolute DL RSTD Measurement Reporting	863

10.1.23.3.2	Differential Reporting for DL RSTD Measurement	865

10.1.23.3.3	Additional Path Report Mapping for DL RSTD	867

10.1.24	PRS-RSRP Measurements	869

10.1.24.1	Introduction	869

10.1.24.2	Measurement Accuracy Requirements	869

10.1.24.2.1	Absolute PRS RSRP accuracy	869

10.1.24.2.2	Relative PRS RSRP accuracy	873

10.1.24.3	Report mapping	877

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	877

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	878

10.1.25	UE Rx-Tx Time Difference Measurements	880

10.1.25.1	Introduction	880

10.1.25.2	Measurement Accuracy Requirements	880

10.1.25.3	Report mapping	890

10.1.25.3.1	Absolute UE Rx-Tx Measurement Report Mapping	890

10.1.25.3.2	Differential UE Rx-Tx Measurement Report Mapping	892

10.1.25.3.3	Additional Path Report Mapping for UE Rx-Tx Time Difference	893

10.1.26	FR2 P-MPR report	895

10.1.26.1	Report mapping	895

10.1.27	L1-SINR accuracy requirements for FR1	895

10.1.27.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	895

10.1.27.1.1	Absolute Accuracy	895

10.1.27.1.2	Relative Accuracy	896

10.1.27.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	897

10.1.27.2.1	Absolute Accuracy	897

10.1.27.2.2	Relative Accuracy	898

10.1.27.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	900

10.1.27.3.1	Absolute Accuracy	900

10.1.27.3.2	Relative Accuracy	902

10.1.28	L1-SINR accuracy requirements for FR2	903

10.1.29	Intra-frequency RSRQ accuracy requirements under CCA	910

10.1.29.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	910

10.1.29.1.1	Absolute SS-RSRQ Accuracy	910

10.1.30	Inter-frequency RSRQ accuracy requirements under CCA	910

10.1.30.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	910

10.1.30.1.1	Aboslute Accuracy of SS-RSRQ	910

10.1.30.1.2	Relative Accuracy of SS-RSRQ	911

10.1.31	Intra-frequency SINR accuracy requirements under CCA	912

10.1.31.1	Intra-frequency SS-SINR accuracy requirements in FR1	912

10.1.31.1.1	Absolute SS-SINR Accuracy	912

10.1.32	Inter-frequency SINR accuracy requirements under CCA	912

10.1.32.1	Inter-frequency SS-SINR accuracy requirements in FR1	912

10.1.32.1.1	Aboslute Accuracy of SS-SINR	912

10.1.32.1.2	Relative Accuracy of SS-SINR	913

10.1.33	L1-RSRP accuracy requirements under CCA	914

10.1.33.1	SSB based L1-RSRP accuracy requirements in FR1	914

10.1.33.1.1	Absolute Accuracy	914

10.1.33.1.2	Relative Accuracy	914

10.1.34	RSSI measurements under CCA	915

10.1.34.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	915

10.1.34.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	915

10.1.34.3	RSSI measurement report mapping	915

10.1.35	Channel occupancy measurements under CCA	916

10.1.35.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	916

10.1.35.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	916

10.1.36	Intra-frequency RSRP accuracy requirements under CCA	916

10.1.36.1	Intra-frequency SS-RSRP accuracy requirements in FR1	916

10.1.36.1.1	Absolute SS-RSRP Accuracy	916

10.1.36.1.2	Relative SS-RSRP Accuracy	917

10.1.37	Inter-frequency RSRP accuracy requirements under CCA	918

10.1.37.1	Inter-frequency SS-RSRP accuracy requirements in FR1	918

10.1.37.1.1	Absolute Accuracy of SS-RSRP	918

10.1.37.1.2	Relative Accuracy of SS-RSRP	918

10.1.38	PRS-RSRPP Measurements	919

10.1.38.1	Introduction	919

10.1.38.2	Measurement Accuracy Requirements	919

10.1.38.2.1	Absolute PRS RSRPP accuracy	919

10.1.38.3	Report mapping	923

10.1.38.3.1	Absolute PRS-RSRPP Measurement Report Mapping	923

10.1.38.3.2	Differential Report Mapping for PRS-RSRPP Measurement	924

10.1.39	UE Rx-Tx time difference measurements for RTT-based PDC	925

10.1.39.1	 Introduction	925

10.1.39.2	 Measurement Accuracy Requirements for PRS	925

10.1.39.3	 Measurement Accuracy Requirements for TRS	929

10.1.40	Measurement Accuracy Requirements for Propagation Delay Compensation	932

10.1.40.1	Measurement Accuracy Requirements for PRS	932

10.1.40.2	Measurement Accuracy Requirements for TRS	935

10.1A	NR measurements for RedCap	938

10.1A.1	Introduction	938

10.1A.2	Intra-frequency RSRP accuracy requirements for FR1	938

10.1A.2.1	Intra-frequency SS-RSRP accuracy requirements	938

10.1A.2.1.1	Absolute SS-RSRP Accuracy	938

10.1A.2.1.2	Relative SS-RSRP Accuracy	939

10.1A.3	Intra-frequency RSRP accuracy requirements for FR2	940

10.1A.3.1	Intra-frequency SS-RSRP accuracy requirements	940

10.1A.3.1.1	Absolute SS-RSRP Accuracy	940

10.1A.3.1.2	Relative SS-RSRP Accuracy	940

10.1A.4	Inter-frequency RSRP accuracy requirements for FR1	940

10.1A.4.1	Inter-frequency SS-RSRP accuracy requirements	940

10.1A.4.1.1	Absolute Accuracy of SS-RSRP in FR1	940

10.1A.4.1.2	Relative Accuracy of SS-RSRP in FR1	941

10.1A.5	Inter-frequency RSRP accuracy requirements for FR2	942

10.1A.5.1	Inter-frequency SS-RSRP accuracy requirements	942

10.1A.5.1.1	Absolute SS-RSRP Accuracy	942

10.1A.5.1.2	Relative SS-RSRP Accuracy	942

10.1A.6	Intra-frequency RSRQ accuracy requirements for FR1	942

10.1A.6.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	942

10.1A.6.1.1	Absolute SS-RSRQ Accuracy in FR1	942

10.1A.7	Intra-frequency RSRQ accuracy requirements for FR2	943

10.1A.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	943

10.1A.7.1.1	Absolute SS-RSRQ Accuracy in FR2	943

10.1A.8	Inter-frequency RSRQ accuracy requirements for FR1	943

10.1A.8.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	943

10.1A.8.1.1	Absolute Accuracy of SS-RSRQ in FR1	943

10.1A.8.1.2	Relative Accuracy of SS-RSRQ in FR1	944

10.1A.9	Inter-frequency RSRQ accuracy requirements for FR2	945

10.1A.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	945

10.1A.9.1.1	Absolute Accuracy of SS-RSRQ in FR2	945

10.1A.9.1.2	Relative Accuracy of SS-RSRQ in FR2	945

10.1A.10	 Intra-frequency SINR accuracy requirements for FR1	945

10.1A.10.1	Intra-frequency SS-SINR accuracy requirements in FR1	945

10.1A.10.1.1	Absolute SS-SINR Accuracy in FR1	945

10.1A.11	Intra-frequency SINR accuracy requirements for FR2	946

10.1A.11.1	Intra-frequency SS-SINR accuracy requirements in FR2	946

10.1A.11.1.1	Absolute SS-SINR Accuracy in FR2	946

10.1A.12 	Inter-frequency SINR accuracy requirements for FR1	946

10.1A.12.1	Inter-frequency SS-SINR accuracy requirements in FR1	946

10.1A.12.1.1	Aboslute Accuracy of SS-SINR in FR1	946

10.1A.12.1.2	Relative Accuracy of SS-SINR in FR1	947

10.1A.13	 Inter-frequency SINR accuracy requirements for FR2	948

10.1A.13.1	Inter-frequency SS-SINR accuracy requirements in FR2	948

10.1A.13.1.1	Aboslute Accuracy of SS-SINR in FR2	948

10.1A.13.1.2	Relative Accuracy of SS-SINR in FR2	948

10.1A.14	 L1-RSRP accuracy requirements for FR1	948

10.1A.14.1	SSB based L1-RSRP accuracy requirements	948

10.1A.14.1.1	Absolute Accuracy	948

10.1A.14.1.2	Relative Accuracy	949

10.1A.14.2	CSI-RS based L1-RSRP accuracy requirements	950

10.1A.14.2.1	Absolute Accuracy	950

10.1A.14.2.2	Relative Accuracy	951

10.1A.15	 L1-RSRP accuracy requirements for FR2	952

10.1A.15.1	SSB based L1-RSRP accuracy requirements	952

10.1A.15.1.1	Absolute Accuracy	952

10.1A.15.1.2	Relative Accuracy	952

10.1A.15.2	CSI-RS based L1-RSRP accuracy requirements	952

10.1A.15.2.1	Absolute Accuracy	952

10.1A.15.2.2	Relative Accuracy	952

10.2	E-UTRAN measurements	953

10.2.1	Introduction	953

10.2.2	E-UTRAN RSRP measurements	953

10.2.3	E-UTRAN RSRQ measurements	953

10.2.4	E-UTRAN RSTD measurements	953

10.2.5	E-UTRAN RS-SINR measurements	954

10.2.6	E-UTRAN RSRP measurements for CA/DC Idle Mode Measurements	954

10.2.7	E-UTRAN RSRQ measurements for CA/DC Idle Mode Measurements	954

10.3	UTRAN FDD Measurements	954

10.3.1	UTRAN FDD CPICH RSCP	955

10.3.2	UTRAN FDD CPICH Ec/No	955

10.4	V2X measurements	956

10.4.1	Introduction	956

10.4.2	Intra-frequency PSBCH-RSRP accuracy requirements for FR1	956

10.4.2.1	PSBCH-RSRP Absolute Accuracy	956

10.4.2.2	PSBCH-RSRP Relative Accuracy	956

10.4.3	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1	957

10.4.3.1	Absolute SL-RSSI Accuracy	957

10.4.4	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1	958

10.4.4.1	Absolute L1 SL-RSRP Accuracy	958

10.4.5	Intra-Frequency Discovery Signal Measurement Accuracy Requirements	958

10.4.5.1	Absolute Discovery Signal Measurement Accuracy	958

11	Void	959

12	V2X Requirements	959

12.1	Introduction	959

12.2	UE Transmit Timing	959

12.2.1	Introduction	959

12.2.2	GNSS as synchronization reference source	959

12.2.3	NR Cell as synchronization reference source	960

12.2.4	E-URTAN Cell as synchronization reference source	960

12.2.5	SyncRef UE as synchronization reference source	960

12.3	Initiation/Cease of SLSS Transmissions	961

12.3.1	Introduction	961

12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	961

12.3.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	962

12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	963

12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	963

12.4	Selection / Reselection of V2X Synchronization Reference Source	964

12.5	L1 SL-RSRP measurements	965

12.5.1	Introduction	965

12.5.2	SL-RSRP measurements	965

12.6	Congestion Control measurements	966

12.7	Interruption	966

12.7.1	Interruptions to WAN due to V2X Sidelink Communication	966

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change	967

12.7.3	Interruptions to WAN due to switching between E-UTRA V2X Sidelink and NR V2X Sidelink	968

12.7.4	Interruptions to WAN at transitions between active and non-active during SL-DRX	969

12.7.5	Interruptions to V2X sidelink at transitions between active and non-active during DRX	969

12.7.6	Interruptions to V2X sidelink due to Active BWP switching Requirement	970

12.7.7	Interruptions to WAN due to SyncRef UE detection and/or Sensing during SL DRX off duration	970

12.7.8	Interruptions at NR sidelink discovery configuration	970

12.8	Reliability of GNSS signal	970

12.9	Scheduling availability	971

12.9.1	Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink	971

12.9.2	Scheduling availability of UE switching between Uu uplink  and V2X sidelink	971

12.10	Selection / Reselection of relay UE	972

12.10.1	Introduction	972

12.10.2	Selection / Reselection of relay UE	972

13	Measurement Performance Requirements for NR gNB	972

13.1	UL-RTOA	972

13.1.1	Report mapping	972

13.1.1A	Additional Path Report Mapping for UL-RTOA	974

13.2	gNB Rx-Tx time difference	976

13.2.1	Report mapping	976

13.2.1A	 Additional Path Report Mapping for gNB Rx-Tx	978

13.2.2	Measurement Accuracy Requirements	980

13.2.2.1	Introduction	980

13.3	UL SRS RSRP measurement	981

13.3.1	Report mapping	981

13.3.2	Measurement accuracy requirements	982

13.3.2.1	Introduction	982

13.3.2.2	Requirements	982

13.4	AoA/ZoA	983

13.4.1	Report mapping	983

13.5	Timing advance (TADV)	984

13.5.1	Report mapping	984

13.6	UL SRS RSRPP measurement	985

13.6.1	Report mapping	985

13.7	gNB Rx-Tx time difference measurements for RTT-based PDC	987

13.7.1	Report mapping	987

13.7.2	Measurement Accuracy Requirements	987

13.7.2.1	Introduction	987

13.7.2.2	Requirements	987

Annex A (normative): Test Cases	967

A.1	Purpose of annex	967

A.2	Requirement classification for statistical testing	967

A.2.1	Types of requirements in TS 38.133	967

A.2.1.1	Time and delay requirements on UE higher layer actions	967

A.2.1.2	Measurements of power levels, relative powers and time	967

A.2.1.3	Implementation requirements	968

A.2.1.4	Physical layer timing requirements	968

A.2.1.5	Requirements under CCA	968

A.3	RRM test configurations	969

A.3.1	Reference measurement channels	969

A.3.1.1	PDSCH	969

A.3.1.1.1	FDD	969

A.3.1.1.2	TDD	970

A.3.1.2	CORESET for RMSI scheduling	973

A.3.1.2.1	FDD	973

A.3.1.2.2	TDD	974

A.3.1.3	CORESET for RMC scheduling	977

A.3.1.3.1	FDD	977

A.3.1.3.2	TDD	978

A.3.1.4	TDD UL/DL configuration	981

A.3.1A	Reference measurement channels under CCA	983

A.3.1A.1	PDSCH	983

A.3.1A.1.1	TDD	983

A.3.1A.2	CORESET for RMSI scheduling	984

A.3.1A.2.1	TDD	984

A.3.1A.3	CORESET for RMC scheduling	985

A.3.1A.3.1	TDD	985

A.3.1A.4	TDD UL/DL configuration	986

A.3.1A.5	RMC burst transmission model	986

A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)	986

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs	987

A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup	987

A.3.2.1.3	OCNG pattern 3: Generic OCNG pattern for unused REs in the same bandwidth as CORESET	988

A.3.2.1.4	OCNG pattern 4: Generic OCNG pattern for all unused REs outside SSB slot(s)	988

A.3.2.2	Void	989

A.3.3	Reference DRX configurations	989

A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms	989

A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms	990

A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity	990

A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms and TAT = Infinity	990

A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms and TAT = Infinity	991

A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = 500 ms	991

A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity	991

A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity	992

A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms	992

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms and TAT = 500 ms	992

A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity	993

A.3.3.12	DRX Configuration 12: DRX cycle = 640 ms and TAT = Infinity	993

A.3.3.13	DRX Configuration X1: DRX cycle = 80 ms and TAT = Infinity	993

A.3.4	Test Cases with Different Channel Bandwidths	994

A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths	994

A.3.4.1.1	Introduction	994

A.3.4.1.2	Principle of testing	994

A.3.5	Test Cases for Synchronous and Asynchronous DC Operations	994

A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	994

A.3.5.1.1	Introduction	994

A.3.5.1.2	Principle of Testing	994

A.3.6	Antenna configurations	994

A.3.6.1	Antenna configurations for FR1	994

A.3.6.1.1	Antenna connection for 4 Rx capable UEs	994

A.3.6.1.1.1	Introduction	994

A.3.6.1.1.2	Principle of testing	995

A.3.6.2	Antenna configurations for FR2	998

A.3.6A	Antenna configurations with unlicensed bands	998

A.3.6A.1	Antenna configurations for FR1	998

A.3.6A.1.1	Antenna connection for 4 Rx capable UEs	998

A.3.6A.1.1.1	Introduction	998

A.3.6A.1.1.2	Principle of testing	998

A.3.7	EN-DC test setup	1000

A.3.7.1	Introduction	1000

A.3.7.2	E-UTRAN Serving Cell Parameters	1000

A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	1000

A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	1001

A.3.7A	NR FR1-FR2 test setup	1002

A.3.7B	EN-DC test setup with unlicensed bands	1003

A.3.7B.1	Introduction	1003

A.3.7B.2	E-UTRAN Serving Cell Parameters	1003

A.3.7B.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	1003

A.3.7C	LTE-FR1/FR2 test setup	1004

A.3.7D	NE-DC test setup	1004

A.3.7D.1	Introduction	1004

A.3.7D.2	E-UTRAN Serving Cell Parameters	1004

A.3.7D.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	1004

A.3.7D.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	1004

A.3.8	PRACH configurations	1004

A.3.8.1	Introduction	1004

A.3.8.2	PRACH configurations in FR1	1005

A.3.8.2.1	FR1 PRACH configuration 1	1005

A.3.8.2.2	FR1 PRACH configuration 2	1005

A.3.8.2.3	FR1 PRACH configuration 3	1006

A.3.8.2.4	FR1 PRACH configuration 4	1007

A.3.8.3	PRACH configurations in FR2	1008

A.3.8.3.1	FR2 PRACH configuration 1	1008

A.3.8.3.2	FR2 PRACH configuration 2	1009

A.3.8.3.3	FR2 PRACH configuration 3	1010

A.3.8.3.4	FR2 PRACH configuration 4	1011

A.3.8A	PRACH configurations under CCA	1012

A.3.8A.1	Introduction	1012

A.3.8A.2	PRACH configurations in FR1	1012

A.3.8A.2.1	FR1 PRACH configuration 1 under CCA	1012

A.3.8A.2.2	FR1 PRACH configuration 2 under CCA	1013

A.3.9	BWP configurations	1014

A.3.9.1	Introduction	1014

A.3.9.2	Downlink BWP configurations	1015

A.3.9.2.1	Initial BWP	1015

A.3.9.2.2	Dedicated BWP	1015

A.3.9.3	Uplink BWP configurations	1016

A.3.9.3.1	Initial BWP	1016

A.3.9.3.2	Dedicated BWP	1016

A.3.9A	BWP configurations for RedCap	1016

A.3.9A.1	Introduction	1016

A.3.9A.2	Downlink BWP configurations	1017

A.3.9A.2.1	Dedicated BWP	1017

A.3.9A.3	Uplink BWP configurations	1017

A.3.9A.3.2	Dedicated BWP	1017

A.3.10	SSB Configurations	1018

A.3.10.1	SSB Configurations for FR1	1018

A.3.10.1.1	SSB pattern 1 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1018

A.3.10.1.5	SSB pattern 5 in FR1: SSB allocation for SSB SCS=15 kHz starting from odd SFN in 10 MHz	1020

A.3.10.1.6	SSB pattern 6 in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 40 MHz	1020

A.3.10.1.7	SSB pattern 7 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1021

A.3.10.1.8	SSB pattern 8 in FR1: SSB allocation for SSB SCS=30 kHz in 40 MHz	1021

A.3.10.2	SSB Configurations for FR2	1022

A.3.10.2.1	SSB pattern 1 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1022

A.3.10.2.2	SSB pattern 2 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1022

A.3.10.2.3	SSB pattern 3 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1023

A.3.10.2.4	SSB pattern 4 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1023

A.3.10.2.5	SSB pattern 5 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1024

A.3.10.2.6	SSB pattern 6 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1024

A.3.10.2.7	SSB pattern 7 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1025

A.3.10.2.8	SSB pattern 8 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1025

A.3.10.2.9	SSB pattern 9 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1026

A.3.10.2.10	SSB pattern 10 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1026

A.3.10A	SSB Configurations under CCA	1031

A.3.10A.1	SSB Configurations under CCA for FR1	1031

A.3.10A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz	1031

A.3.10A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz	1031

A.3.10A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1032

A.3.10A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1032

A.3.10B	SSB Configurations for RedCap	1033

A.3.10B.1	SSB Configurations for FR1	1033

A.3.10B.1.1	SSB pattern 1 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1033

A.3.10B.1.2	SSB pattern 2 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1033

A.3.10B.1.3	SSB pattern 3 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 20 MHz	1034

A.3.10B.1.4	SSB pattern 4 for RedCap in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1034

A.3.10B.1.5	SSB pattern 5 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1035

A.3.10B.1.6	SSB pattern 6 for RedCap in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1035

A.3.10B.1.7	SSB pattern 7 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1036

A.3.10B.2	SSB Configurations for FR2	1036

A.3.10B.2.1	SSB pattern 1 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1036

A.3.10B.2.2	SSB pattern 2 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1037

A.3.10B.2.3	SSB pattern 3 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1037

A.3.10B.2.4	SSB pattern 4 for RedCap in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1038

A.3.10B.2.5	SSB pattern 5 for RedCap in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1038

A.3.11	SMTC Configurations	1039

A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms	1039

A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms	1039

A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms	1039

A.3.11.4	SMTC pattern 4: SMTC period = 20 ms with SMTC duration = 1 ms	1039

A.3.11.5	SMTC pattern 5: SMTC period = 20 ms with SMTC duration = 5 ms	1040

A.3.11.6	SMTC pattern 6: SMTC period = 20 ms with SMTC duration = 5 ms	1040

A.3.11.7	SMTC pattern 7: SMTC period = 20 ms with SMTC duration = 5 ms	1040

A.3.11A	SMTC Configurations for RedCap	1040

A.3.11A.1	SMTC pattern 1 for RedCap: SMTC period = 40 ms with SMTC duration = 1 ms	1040

A.3.11A.2	SMTC pattern 2 for RedCap: SMTC period = 80 ms with SMTC duration = 1 ms	1040

A.3.11A.3	SMTC pattern 3 for RedCap: SMTC period = 40 ms with SMTC duration = 1 ms	1041

A.3.11A.4	SMTC pattern 4 for RedCap: SMTC period = 80 ms with SMTC duration = 5 ms	1041

A.3.12	Test Cases with Different CC Configurations	1041

A.3.12.1 EN-DC Test Cases with Different EN-DC Configurations	1041

A.3.12.1.1	Introduction	1041

A.3.12.1.2	Principle of testing	1041

A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations	1041

A.3.12.2.1	Introduction	1041

A.3.12.2.2	Principle of testing	1041

A.3.13	Test Cases in SA and EN-DC Operations	1042

A.3.13.1	Introduction	1042

A.3.13.2	Principle of Testing	1042

A.3.13A	Test Cases involving E-UTRA/FR1 and FR2 carriers	1042

A.3.13A.1	Introduction	1042

A.3.13A.2	Principle of Testing in EN-DC	1042

A.3.13A.3	Principle of Testing in SA	1043

A.3.13A.4	Principle of Testing in E-UTRA	1043

A.3.13B	Test Cases for EN-DC and NE-DC Operations	1044

A.3.13B.1	Active BWP switch Test Cases for EN-DC and NE-DC Operations	1044

A.3.13B.1.1	Introduction	1044

A.3.13B.1.2	Principle of Testing	1044

A.3.13B.2	SFTD accuracy Test Cases for EN-DC and NE-DC Operations	1044

A.3.13B.2.1	Introduction	1044

A.3.13B.2.2	Principle of Testing	1044

A.3.14	CSI-RS configurations	1045

A.3.14.1	FDD	1045

A.3.14.2	TDD	1047

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases	1053

A.3.15.1	Setup 1: Single AoA in Rx beam peak direction	1053

A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction	1053

A.3.15.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	1053

A.3.15.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	1054

A.3.15.3	Setup 3: 2 AoAs	1054

A.3.15.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	1054

A.3.15.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	1054

A.3.15.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	1054

A.3.16	TCI State Configuration	1055

A.3.16.1	Introduction	1055

A.3.16.2	TCI states	1055

A.3.16A	Unified TCI State Configuration	1055

A.3.16A.1	Introduction	1055

A.3.16A.2	DLorJoint TCI states	1056

A.3.16A.3	UL TCI states	1056

A.3.17	Configurations of CSI-RS for tracking	1057

A.3.17.1	Configuration of CSI-RS for tracking for FR1	1057

A.3.17.1.1	FDD	1057

A.3.17.1.2	TDD	1059

A.3.17.2	Configuration of CSI-RS for tracking for FR2	1061

A.3.17.2.1	TDD	1061

A.3.18	Additional definitions related to OTA testing for FR2 RRM test cases	1062

A.3.18.1	Introduction	1062

A.3.18.2	PRACH Power Measurement	1062

A.3.19	Test applicability for DAPS handover	1062

A.3.19.1	Introduction	1062

A.3.19.2	Principle of testing	1062

A.3.20	MsgA configurations	1063

A.3.20.1	Introduction	1063

A.3.20.2	MsgA configurations in FR1	1063

A.3.20.2.1	FR1 MsgA configuration 1	1063

A.3.20.2.2	FR1 MsgA configuration 2	1064

A.3.20.3	MsgA configurations in FR2	1066

A.3.20.3.1	FR2 MsgA configuration 1	1066

A.3.20.3.2	FR2 MsgA configuration 2	1067

A.3.20A	MsgA configurations under CCA	1069

A.3.20A.1	Introduction	1069

A.3.20A.2	MsgA configurations in FR1	1069

A.3.20A.2.1	FR1 MsgA configuration 1 under CCA	1069

A.3.20A.2.2	FR1 MsgA configuration 2 under CCA	1070

A.3.21	V2X sidelink communication	1073

A.3.21.1	Introduction	1073

A.3.21.2	Reference resource pool configurations for V2X Sidelink Communication	1073

A.3.21.3	Reference measurement channels for V2X Sidelink Communication	1077

A.3.21.4	Reference SL-DRX configurations	1078

A.3.21.4.1	SL-DRX Configuration 1: SL-DRX cycle = 40 ms	1078

A.3.21.4.2	SL-DRX Configuration 2: SL-DRX cycle = 320 ms	1078

A.3.21.4.3	SL-DRX Configuration 3: SL-DRX cycle = 640 ms	1078

A.3.22	CSI-IM configurations	1078

A.3.22.1	FDD	1078

A.3.22.2	TDD	1079

A.3.23	Spatial Relation Configuration	1080

A.3.23.1	Introduction	1080

A.3.23.2	Spatial Relation	1081

A.3.24	SRS configuration	1082

A.3.25	Channel bandwidth (CBW) configurations	1084

A.3.25.1	DL UE specific CBW	1084

A.3.25.2	UL UE specific CBW	1085

A.3.26	CCA model	1085

A.3.26.1	Introduction	1085

A.3.26.2	CCA model for operation on a carrier frequency with CCA in FR1	1085

A.3.26.2.1	DL CCA model	1085

A.3.26.2.2	UL CCA model	1086

A.3.26.3	CCA model for operation on a carrier frequency with CCA in FR2-2	1087

A.3.26.3.1	DL CCA model	1087

A.3.26.3.2	UL CCA model	1088

A.3.27	Test Cases with at Least One Cell on a Carrier Frequency with CCA	1088

A.3.27.1	Introduction	1088

A.3.27.2	 NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	1088

A.3.27.3	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	1088

A.3.27.4	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	1089

A.3.27.5	E-UTRA Standalone Tests with at Least One NR Cell under CCA	1089

A.3.28	Discovery Burst Transmission Window configuration under CCA	1089

A.3.28.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	1089

A.3.29	Testing principles for UE capable of only NR bands with shared spectrum access	1089

A.3.29.1	Introduction	1089

A.3.29.2	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	1089

A.3.29.3	Principle of testing for UE capable of SA operation with only NR bands with shared spectrum access	1090

A.3.30	CSI-RS configurations for RRM	1090

A.3.30.1	FDD	1090

A.3.30.2	TDD	1091

A.3.31	PRS Configurations	1093

A.3.31.1.	PRS Configurations for FR1	1093

A.3.31.1.1.	PRS pattern 1 in FR1: SCS=15 KHz	1093

A.3.31.1.2.	PRS pattern 2 in FR1: SCS=30 KHz	1094

A.3.31.2.	PRS Configurations for FR2	1094

A.3.31.2.1.	PRS pattern 1 in FR2: SCS=120 KHz	1094

A.3.32	NR sidelink discovery	1094

A.3.32.1	Introduction	1094

A.3.32.2	Reference resource pool configurations for NR Sidelink Discovery	1094

A.3.32.3	Principle of Testing	1095

A.3.33	PRS Processing Window (PPW) configurations	1095

A.3.34	Testing principles for test cases related to PRS measurements	1095

A.3.34.1	Introduction	1095

A.3.34.2	Test cases in RRC_INACTIVE state	1096

A.3.34.3	Test cases for PRS measurements with gaps in RRC_CONNECTED state	1096

A.3.34.4	Test cases for PRS measurements without gaps in RRC_CONNECTED state	1096

A.3.35	Testing principle for RedCap UE	1097

A.3.35.1	Introduction	1097

A.3.35.2	Principle of testing for FR1	1097

A.3.35.3	Principle of testing for FR2	1097

A.3.36	Testing related to Satellite access	1097

A.3.36.1	Introduction	1097

A.3.36.2	Principle of testing GSO and NGSO scenarios	1097

A.3.36.2	Principle of testing different RRM requirements	1098

A.3.36.3	Principle of testing different ephemeris formats	1098

A.3.36.4	General setup for SIB19	1100

A.4	EN-DC tests with all NR cells in FR1	1096

A.4.1	Void	1096

A.4.2	Void	1096

A.4.3	RRC_CONNECTED state mobility	1096

A.4.3.1	Void	1096

A.4.3.2	RRC Connection Mobility Control	1096

A.4.3.2.1	Void	1096

A.4.3.2.2	Random Access	1096

A.4.3.2.2.1	4-step RA type contention based random access test in FR1 for PSCell in EN-DC	1096

A.4.3.2.2.2	4-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	1100

A.4.3.2.2.3	2-step RA type contention based random access test in FR1 for PSCell in EN-DC	1104

A.4.3.2.2.4	2-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	1108

A.4.3.2.3	Void	1112

A.4.3.3	Handover with PSCell from EN-DC to EN-DC with known target PSCell in FR1	1112

A.4.3.3.1	Test Purpose and Environment	1112

A.4.3.3.2	Test Requirements	1118

A.4.4	Timing	1119

A.4.4.1	UE transmit timing	1119

A.4.4.1.1	NR UE Transmit Timing Test for FR1	1119

A.4.4.1.1.1	Test Purpose and environment	1119

A.4.4.1.1.2	Test requirements	1123

A.4.4.2	UE timer accuracy	1123

A.4.4.3	Timing advance	1123

A.4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	1123

A.4.4.3.1.1	Test Purpose and Environment	1123

A.4.4.3.1.2	Test Parameters	1123

A.4.4.3.1.3	Test Requirements	1127

A.4.5	Signaling characteristics	1127

A.4.5.1	Radio link Monitoring	1127

A.4.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	1128

A.4.5.1.1.1	Test Purpose and Environment	1128

A.4.5.1.1.2	Test Requirements	1132

A.4.5.1.2	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	1132

A.4.5.1.2.1	Test Purpose and Environment	1132

A.4.5.1.2.2	Test Requirements	1138

A.4.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	1138

A.4.5.1.3.1	Test Purpose and Environment	1138

A.4.5.1.3.2	Test Requirements	1143

A.4.5.1.4	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	1143

A.4.5.1.4.1	Test Purpose and Environment	1143

A.4.5.1.4.2	Test Requirements	1149

A.4.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	1149

A.4.5.1.5.1	Test Purpose and Environment	1149

A.4.5.1.5.2	Test Requirements	1154

A.4.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	1154

A.4.5.1.6.1	Test Purpose and Environment	1154

A.4.5.1.6.2	Test Requirements	1160

A.4.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	1160

A.4.5.1.7.1	Test Purpose and Environment	1160

A.4.5.1.7.2	Test Requirements	1165

A.4.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	1166

A.4.5.1.8.1	Test Purpose and Environment	1166

A.4.5.1.8.2	Test Requirements	1171

A.4.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	1171

A.4.5.1.9.1	Test Purpose and Environment	1171

A.4.5.2	Interruption	1177

A.4.5.2.1	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1177

A.4.5.2.1.1	Test Purpose and Environment	1177

A.4.5.2.1.2	Test Requirements	1181

A.4.5.2.2	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1181

A.4.5.2.2.1	Test Purpose and Environment	1181

A.4.5.2.2.2	Test Requirements	1185

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1185

A.4.5.2.3.1	Test Purpose and Environment	1185

A.4.5.2.3.2	Test Requirements	1190

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1191

A.4.5.2.4.1	Test Purpose and Environment	1191

A.4.5.2.4.2	Test Requirements	1195

A.4.5.2.5	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1196

A.4.5.2.5.1	Test Purpose and Environment	1196

A.4.5.2.5.2	Test Requirements	1200

A.4.5.2.6	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1200

A.4.5.2.6.1	Test Purpose and Environment	1200

A.4.5.2.6.2	Test Requirements	1204

A.4.5.2.7	Void	1204

A.4.5.2.8	E-UTRAN - NR FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	1204

A.4.5.2.8.1	Test Purpose and Environment	1204

A.4.5.2.8.2	Test Requirements	1207

A.4.5.2.9	E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching	1208

A.4.5.2.9.1	Test Purpose and Environment	1208

A.4.5.2.9.2	Test Requirements	1211

A.4.5.2.10	E-UTRAN – NR FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell	1212

A.4.5.2.10.1	Test Purpose and Environment	1212

A.4.5.2.10.2	Test Requirements	1215

A.4.5.2.11	E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching with 1 SRS symbol in a slot in synchronous EN-DC	1215

A.4.5.2.11.1	Test Purpose and Environment	1215

A.4.5.2.11.2	Test Requirements	1220

A.4.5.2.12	E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching in asynchronous EN-DC	1221

A.4.5.2.12.1	Test Purpose and Environment	1221

A.4.5.3	SCell Activation and Deactivation Delay	1228

A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1228

A.4.5.3.1.1	Test Purpose and Environment	1228

A.4.5.3.1.2	Test Requirements	1234

A.4.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 640ms SCell measurement cycle	1235

A.4.5.3.2.1	Test Purpose and Environment	1235

A.4.5.3.2.2	Test Requirements	1235

A.4.5.3.3	SCell Activation and deactivation of unknown SCell in FR1	1235

A.4.5.3.3.1	Test Purpose and Environment	1235

A.4.5.3.3.2	Test Requirements	1236

A.4.5.3.4	SCell Activation and deactivation of multiple unknown SCells in FR1 with single activation/deactivation command	1237

A.4.5.3.4.1	Test Purpose and Environment	1237

A.4.5.3.3.2	Test Requirements	1239

A.4.5.3.5	Direct SCell activation at SCell addition of known SCell in FR1	1239

A.4.5.3.5.1	Test Purpose and Environment	1239

A.4.5.3.5.2	Test Requirements	1247

A.4.5.3.3	Fast SCell Activation of known SCell in FR1 for 160ms SCell measurement cycle	1248

A.4.5.3.3.1	Test Purpose and Environment	1248

A.4.5.3.3.2	Test Requirements	1252

A.4.5.3.4	Fast SCell Activation of known SCell in FR1 for 640 ms SCell measurement cycle	1253

A.4.5.3.4.1	Test Purpose and Environment	1253

A.4.5.3.4.2	Test Requirements	1253

A.4.5.4	UE UL carrier RRC reconfiguration Delay	1254

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay	1254

A.4.5.4.1.1 Test Purpose and Environment	1254

A.4.5.4.1.2	Test Requirements	1264

A.4.5.5	Beam Failure Detection and Link recovery procedures	1264

A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	1264

A.4.5.5.1.1	Test Purpose and Environment	1264

A.4.5.5.1.2	Test Requirements	1270

A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	1270

A.4.5.5.2.1	Test Purpose and Environment	1270

A.4.5.5.2.2	Test Requirements	1276

A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1276

A.4.5.5.3.1	Test Purpose and Environment	1276

A.4.5.5.3.2	Test Requirements	1282

A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1282

A.4.5.5.4.1	Test Purpose and Environment	1282

A.4.5.5.4.2	Test Requirements	1288

A.4.5.5.5	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1288

A.4.5.5.5.1	Test Purpose and Environment	1288

A.4.5.5.5.2	Test Requirements	1294

A.4.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	1294

A.4.5.5.6.1	Test Purpose and Environment	1294

A.4.5.5.6.2	Test Requirements	1301

A.4.5.5.7	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	1301

A.4.5.5.7.1	Test Purpose and Environment	1301

A.4.5.5.7.2	Test Requirements	1307

A.4.5.5.8	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1307

A.4.5.5.8.1	Test Purpose and Environment	1307

A.4.5.5.8.2	Test Requirements	1314

A.4.5.6	Active BWP switch	1315

A.4.5.6.1	DCI-based and Timer-based Active BWP Switch	1315

A.4.5.6.1.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	1315

A.4.5.6.1.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	1320

A.4.5.6.2	RRC-based Active BWP Switch	1326

A.4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1330

A.4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	1330

A.4.5.6.3.1.1	Test Purpose and Environment	1330

A.4.5.6.3.1.2	Test Requirements	1336

A.4.5.6.4	Simultaneous RRC-based Active BWP Switch on multiple CCs	1336

A.4.5.6.4.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	1336

A.4.5.6.4.1.1	Test Purpose and Environment	1336

A.4.5.6.4.1.2	Test Requirements	1343

A.4.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	1343

A.4.5.6.4.2.1	Test Purpose and Environment	1343

A.4.5.6.4.2.2	Test Requirements	1353

A.4.5.6.5	SCell dormancy switch	1354

A.4.5.6.5.1	E-UTRAN – NR FR1 PSCell SCell dormancy switch of single FR1 SCell outside active time	1354

A.4.5.6.5.1.1	Test Purpose and Environment	1354

A.4.5.6.5.1.2	Test Requirements	1359

A.4.5.6.5.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	1359

A.4.5.6.5.2.1	Test Purpose and Environment	1359

A.4.5.6.5.2.2	Test Requirements	1365

A.4.5.7	PSCell addition and release delay	1365

A.4.5.7.1	Addition and Release Delay of known NR PSCell	1365

A.4.5.7.1.1	Test purpose and environment	1365

A.4.5.7.1.2	Test Requirements	1370

A.4.5.8	DL Interruptions at switching between two uplink carriers	1371

A.4.5.8.1	Test Purpose and Environment	1371

A.4.5.8.2	Test Requirements	1374

A.4.5.9	UE specific CBW change	1375

A.4.5.9.1	UE specific CBW change on FR1 NR PSCell with non-DRX in synchronous EN- DC	1375

A.4.5.9.1.1	Test Purpose and Environment	1375

A.4.5.9.1.2	Test Requirements	1379

A.4.5.10	PSCell activation and deactivation delay	1379

A.4.5.10.1	PSCell activation and deactivation delay	1379

A.4.5.10.1.1	Test purpose and environment	1379

A.4.5.9.1.2	Test Requirements	1383

A.4.5.11	Conditional PSCell addition and release delay (FR1 EN-DC)	1384

A.4.5.11.1	Addition and Release Delay of PSCell	1384

A.4.5.11.1.1	Test purpose and environment	1384

A.4.5.11.1.2	Test Parameters	1384

A.4.5.11.1.3	Test Requirements	1388

A.4.6	Measurement procedure	1388

A.4.6.1	Intra-frequency Measurements	1388

A.4.6.1.1	EN-DC event triggered reporting tests without gap under non-DRX	1388

A.4.6.1.1.1	Test purpose and Environment	1388

A.4.6.1.1.2	Test parameters	1388

A.4.6.1.1.3	Test Requirements	1392

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX	1392

A.4.6.1.2.1	Test purpose and Environment	1392

A.4.6.1.2.2	Test parameters	1392

A.4.6.1.2.2	Test Requirements	1396

A.4.6.1.3	EN-DC event triggered reporting tests with per-UE gaps under non-DRX	1396

A.4.6.1.3.1	Test purpose and Environment	1396

A.4.6.1.3.2	Test parameters	1396

A.4.6.1.3.3	Test Requirements	1400

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX	1400

A.4.6.1.4.1	Test purpose and Environment	1400

A.4.6.1.4.2	Test parameters	1400

A.4.6.1.4.3	Test Requirements	1404

A.4.6.1.5	EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading	1404

A.4.6.1.5.1	Test purpose and Environment	1404

A.4.6.1.5.2	Test parameters	1404

A.4.6.1.5.3	Test Requirements	1406

A.4.6.1.6	EN-DC event triggered reporting tests with SSB index reading with per-UE gaps	1407

A.4.6.1.6.1	Test purpose and Environment	1407

A.4.6.1.6.2	Test parameters	1407

A.4.6.1.6.3	Test Requirements	1409

A.4.6.1.7	EN-DC event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	1410

A.4.6.1.7.1	Test purpose and Environment	1410

A.4.6.1.7.2	Test parameters	1410

A.4.6.1.7.3	Test Requirements	1414

A.4.6.1.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used for UE configured with highSpeedMeasCA-Scell-r17	1414

A.4.6.1.8.1	Test Purpose and Environment	1414

A.4.6.1.8.2	Test Requirements	1419

A.4.6.2	Inter-frequency Measurements	1420

A.4.6.2.1	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	1420

A.4.6.2.1.1	Test Purpose and Environment	1420

A.4.6.2.1.2	Test Requirements	1424

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	1424

A.4.6.2.2.1	Test Purpose and Environment	1424

A.4.6.2.2.2	Test Requirements	1428

A.4.6.2.3	Void	1429

A.4.6.2.4	Void	1429

A.4.6.2.5	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	1429

A.4.6.2.5.1	Test Purpose and Environment	1429

A.4.6.2.5.2	Test Requirements	1433

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	1433

A.4.6.2.6.1	Test Purpose and Environment	1433

A.4.6.2.6.2	Test Requirements	1438

A.4.6.2.7	Void	1439

A.4.6.2.8	Void	1439

A.4.6.2.9	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used for UE configured with highSpeedMeasInterFreq-r17	1439

A.4.6.2.9.1	Test Purpose and Environment	1439

A.4.6.2.9.2	Test Requirements	1442

A.4.6.3	Void	1443

A.4.6.4	L1-RSRP measurement for beam reporting	1443

A.4.6.4.1	SSB based L1-RSRP measurement when DRX is not used	1443

A.4.6.4.1.1	Test Purpose and Environment	1443

A.4.6.4.1.2	Test parameters	1443

A.4.6.4.1.3	Test Requirements	1446

A.4.6.4.2	SSB based L1-RSRP measurement when DRX is used	1446

A.4.6.4.2.1	Test Purpose and Environment	1446

A.4.6.4.2.2	Test parameters	1447

A.4.6.4.2.3	Test Requirements	1450

A.4.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	1450

A.4.6.4.3.1	Test Purpose and Environment	1450

A.4.6.4.3.2	Test parameters	1451

A.4.6.4.3.3	Test Requirements	1454

A.4.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	1454

A.4.6.4.4.1	Test Purpose and Environment	1454

A.4.6.4.4.2	Test parameters	1455

A.4.6.4.4.3	Test Requirements	1457

A.4.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	1457

A.4.6.4.5.1	Test Purpose and Environment	1457

A.4.6.4.5.2	Test parameters	1458

A.4.6.4.5.3	Test Requirements	1461

A.4.6.5	CLI measurements	1461

A.4.6.5.1	SRS-RSRP measurement with non-DRX	1461

A.4.6.5.1.1	Test Purpose and Environment	1461

A.4.6.5.1.2	Test Parameters	1462

A.4.6.5.1.3	Test Requirements	1465

A.4.6.5.2	CLI-RSSI measurement with non-DRX	1465

A.4.6.5.2.1	Test Purpose and Environment	1465

A.4.6.5.2.2	Test Parameters	1466

A.4.6.5.2.3	Test Requirements	1467

A.4.6.6.1.2	Test Requirements	1472

A.4.6.7	L1-SINR measurement for beam reporting	1472

A.4.6.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	1475

A.4.6.7.2.1	Test Purpose and Environment	1475

A.4.6.7.2.2	Test parameters	1475

A.4.6.7.2.3	Test Requirements	1478

A.4.6.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	1478

A.4.6.7.3.1	Test Purpose and Environment	1478

A.4.6.7.3.2	Test parameters	1478

A.4.6.7.3.3	Test Requirements	1480

A.4.6.8	CSI-RS based intra-frequency Measurement	1481

A.4.6.8.1	EN-DC event triggered reporting tests without gap under DRX	1481

A.4.6.8.1.1	Test purpose and Environment	1481

A.4.6.8.1.2	Test Requirements	1485

A.4.6.9	CSI-RS based inter-frequency Measurement	1485

A.4.6.9.1	EN-DC event triggered reporting tests for FR1 cell when non-DRX is used	1485

A.4.6.9.1.1	Test Purpose and Environment	1485

A.4.6.9.1.2	Test Requirements	1489

A.4.7	Measurement Performance requirements	1490

A.4.7.1	SS-RSRP	1490

A.4.7.1.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1490

A.4.7.1.1.1	Test Purpose and Environment	1490

A.4.7.1.1.2	Test parameters	1490

A.4.7.1.1.3	Test Requirements	1494

A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1494

A.4.7.1.2.1	Test Purpose and Environment	1494

A.4.7.1.2.2	Test parameters	1495

A.4.7.1.2.3	Test Requirements	1499

A.4.7.1.3	Void	1499

A.4.7.2	SS-RSRQ	1499

A.4.7.2.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1499

A.4.7.2.1.1	Test Purpose and Environment	1499

A.4.7.2.1.2	Test Parameters	1500

A.4.7.2.1.3	Test Requirements	1505

A.4.7.2.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1505

A.4.7.2.2.1	Test Purpose and Environment	1505

A.4.7.2.2.2	Test Parameters	1505

A.4.7.2.2.3	Test Requirements	1510

A.4.7.3	SS-SINR	1510

A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1510

A.4.7.3.1.1	Test Purpose and Environment	1510

A.4.7.3.1.2	Test Parameters	1510

A.4.7.3.1.3	Test Requirements	1514

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1514

A.4.7.3.2.1	Test Purpose and Environment	1514

A.4.7.3.2.2	Test Parameters	1514

A.4.7.3.2.3	Test Requirements	1519

A.4.7.4	L1-RSRP measurement for beam reporting	1519

A.4.7.4.1	SSB based L1-RSRP measurement	1519

A.4.7.4.1.1	Test Purpose and Environment	1519

A.4.7.4.1.2	Test parameters	1520

A.4.7.4.1.3	Test Requirements	1523

A.4.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1523

A.4.7.4.2.1	Test Purpose and Environment	1523

A.4.7.4.2.2	Test parameters	1524

A.4.7.4.2.3	Test Requirements	1527

A.4.7.5	SFTD accuracy	1527

A.4.7.5.1	SFTD accuracy	1527

A.4.7.5.1.1	Test Purpose and Environment	1527

A.4.7.5.1.2	Test Parameters	1527

A.4.7.5.1.3	Test Requirements	1532

A.4.7.5.2	Void	1532

A.4.7.5.3	Void	1532

A.4.7.6	CLI measurements	1532

A.4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	1532

A.4.7.6.1.1	Test Purpose and Environment	1532

A.4.7.6.1.2	Test parameters	1533

A.4.7.6.1.3	Test Requirements	1538

A.4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	1538

A.4.7.6.2.1	Test Purpose and Environment	1538

A.4.7.6.2.2	Test parameters	1539

A.4.7.6.2.3	Test Requirements	1542

A.4.7.7	L1-SINR measurement for beam reporting	1542

A.4.7.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1546

A.4.7.7.2.1	Test Purpose and Environment	1546

A.4.7.7.2.2	Test parameters	1546

A.4.7.7.2.3	Test Requirements	1550

A.4.7.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1550

A.4.7.7.3.1	Test Purpose and Environment	1550

A.4.7.7.3.2	Test parameters	1550

A.4.7.7.3.3	Test Requirements	1553

A.4.7.8	CSI-RSRP	1553

A.4.7.8.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1553

A.4.7.8.1.1	Test Purpose and Environment	1553

A.4.7.8.1.2	Test parameters	1554

A.4.7.8.1.3	Test Requirements	1559

A.4.7.8.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1559

A.4.7.8.2.1	Test Purpose and Environment	1559

A.4.7.8.2.2	Test parameters	1559

A.4.7.8.2.3	Test Requirements	1564

A.4.7.9	CSI-RSRQ	1564

A.4.7.9.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1564

A.4.7.9.1.1	Test Purpose and Environment	1564

A.4.7.9.1.2	Test Parameters	1564

A.4.7.9.1.3	Test Requirements	1569

A.4.7.9.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1569

A.4.7.9.2.1	Test Purpose and Environment	1569

A.4.7.9.2.2	Test Parameters	1569

A.4.7.9.2.3	Test Requirements	1574

A.4.7.10	CSI-SINR	1574

A.4.7.10.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1574

A.4.7.10.1.1	Test Purpose and Environment	1574

A.4.7.10.1.2	Test Parameters	1575

A.4.7.10.1.3	Test Requirements	1579

A.4.7.10.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1579

A.4.7.10.2.1	Test Purpose and Environment	1579

A.4.7.10.2.2	Test Parameters	1580

A.4.7.10.2.3	Test Requirements	1584

A.4.8	Void	1584

A.4A	NE-DC test with all NR cells in FR1	1585

A.4A.1	Signaling characteristics	1585

A.4A.1.1	E-UTRAN PSCell addition	1585

A.4A.1.1.1	Test purpose and environment	1585

A.4A.1.1.2	Test Requirements	1589

A.4A.1.2	Active BWP switch	1590

A.4A.1.2.1	E-UTRAN PSCell – NR PCell FR1 DCI-based and Timer-based DL active BWP switch in non-DRX in synchronous NE-DC	1590

A.4A.1.2.1.1	Test Purpose and Environment	1590

A.4A.1.2.1.2	Test Requirements	1594

A.4A.1.3	Intra-frequency handover with E-UTRAN PSCell	1595

A.4A.1.3.1	Test purpose and environment	1595

A.4A.1.3.2	Test Requirements	1599

A.4A.1.4	Handover with PSCell from NE-DC to NE-DC with unknown target PSCell	1600

A.4A.1.4.1	Test Purpose and Environment	1600

A.4A.1.4.2	Test Parameters	1600

A.4A.1.4.3	Test Requirements	1605

A.4A.1.4.3.1	Test Requirements for NR HO	1605

A.4A.1.4.3.2	Test Requirements for LTE PSCell Change	1605

A.4A.2	Measurement performance	1606

A.4A.2.1	SFTD accuracy	1606

A.4A.2.1.1	SFTD accuracy	1606

A.4A.2.1.1.1	Test Purpose	1606

A.4A.2.1.1.2	Test Environment	1606

A.4A.2.1.1.3	Test Requirements	1609

A.5	EN-DC tests with one or more NR cells in FR2	1610

A.5.1	Void	1610

A.5.2	Void	1610

A.5.3	RRC_CONNECTED state mobility	1610

A.5.3.1	Void	1610

A.5.3.2	RRC Connection Mobility Control	1610

A.5.3.2.1	Void	1610

A.5.3.2.2	Random Access	1610

A.5.3.2.2.1	4-step RA type c ontention based random access test in FR2 for PSCell/SCell in EN-DC	1610

A.5.3.2.2.2	4-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	1613

A.5.3.2.2.3	2-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	1619

A.5.3.2.2.4	2-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	1622

A.5.3.2.3	Void	1626

A.5.3.3	Handover with PSCell with known FR2 target PSCell	1626

A.5.3.3.1	Test purpose and environment	1626

A.5.3.3.2	Test Requirements	1630

A.5.5.3.3	PUCCH SCell Activation and deactivation of known SCell in FR2	1631

A.5.5.3.3.1	Test Purpose and Environment	1631

A.5.5.3.3.2	Test Requirements	1634

A.5.5.3.4	PUCCH SCell Activation and deactivation of unknown SCell in FR2	1635

A.5.5.3.4.1	Test Purpose and Environment	1635

A.5.5.3.4.2	Test Requirements	1638

A.5.5.3.5	Multiple SCell activation and deactivation of one known PUCCH SCell and one unknown SCell in FR2	1639

A.5.5.3.5.1	Test Purpose and Environment	1639

A.5.5.3.5.2	Test Requirements	1642

A.5.5.3.6	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR2 in non-DRX	1643

A.5.5.3.6.1	Test Purpose and Environment	1643

A.5.5.3.6.2	Test Requirements	1646

A.5.4	Timing	1647

A.5.4.1	UE transmit timing	1647

A.5.4.1.1	NR UE Transmit Timing Test for FR2	1647

A.5.4.1.1.1	Test Purpose and environment	1647

A.5.4.1.1.2	Test requirements	1650

A.5.4.2	UE timer accuracy	1651

A.5.4.3	Timing advance	1651

A.5.4.3.1 EN-DC FR2 timing advance adjustment accuracy	1651

A.5.4.3.1.1 Test Purpose and Environment	1651

A.5.4.3.1.2 Test Parameters	1651

A.5.4.3.1.3	Test Requirements	1655

A.5.5	Signaling characteristics	1655

A.5.5.1	Radio link Monitoring	1655

A.5.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	1655

A.5.5.1.1.1	Test Purpose and Environment	1655

A.5.5.1.1.2	Test Requirements	1659

A.5.5.1.2	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	1660

A.5.5.1.2.1	Test Purpose and Environment	1660

A.5.5.1.2.2	Test Requirements	1663

A.5.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	1664

A.5.5.1.3.1	Test Purpose and Environment	1664

A.5.5.1.3.2	Test Requirements	1668

A.5.5.1.4	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	1668

A.5.5.1.4.1	Test Purpose and Environment	1668

A.5.5.1.4.2	Test Requirements	1672

A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	1672

A.5.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	1676

A.5.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1680

A.5.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1685

A.5.5.1.8.2	Test Requirements	1689

A.5.5.1.9	EN-DC Radio Link Monitoring UE Scheduling Restrictions on FR2	1690

A.5.5.1.9.1	Test Purpose and Environment	1690

A.5.5.1.9.2	Test Requirements	1692

A.5.5.1.10	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	1692

A.5.5.1.10.1	Test Purpose and Environment	1692

A.5.5.1.10.2	Test Requirements	1695

A.5.5.2	Interruption	1695

A.5.5.2.1	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1695

A.5.5.2.1.1	Test Purpose and Environment	1695

A.5.5.2.1.2	Test Requirements	1698

A.5.5.2.2	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1698

A.5.5.2.2.1	Test Purpose and Environment	1698

A.5.5.2.2.2	Test Requirements	1701

A.5.5.2.3	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1701

A.5.5.2.3.1	Test Purpose and Environment	1701

A.5.5.2.3.2	Test Requirements	1705

A.5.5.2.4	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1705

A.5.5.2.4.1	Test Purpose and Environment	1705

A.5.5.2.4.2	Test Requirements	1708

A.5.5.2.5	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1709

A.5.5.2.5.1	Test Purpose and Environment	1709

A.5.5.2.5.2	Test Requirements	1712

A.5.5.2.6	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1712

A.5.5.2.6.1	Test Purpose and Environment	1712

A.5.5.2.6.2	Test Requirements	1715

A.5.5.2.7	E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching	1715

A.5.5.2.7.1	Test Purpose and Environment	1715

A.5.5.2.7.2	Test Requirements	1719

A.5.5.2.8 E-UTRAN – NR FR2 interruptions at NR SRS carrier based switching	1719

A.5.5.2.8.1 Test Purpose and Environment	1719

A.5.5.2.8.3	Test Requirements	1721

A.5.5.3	SCell Activation and Deactivation Delay	1721

A.5.5.3.1	SCell Activation and deactivation of SCell in FR2 intra-band	1721

A.5.5.3.1.1	Test Purpose and Environment	1721

A.5.5.3.1.2	Test Requirements	1723

A.5.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1723

A.5.5.3.2.1	Test Purpose and Environment	1723

A.5.5.3.2.2	Test Requirements	1727

A.5.5.3.3	Void	1727

A.5.5.3.4	Void	1727

A.5.5.3.5	SCell Activation and deactivation of SCell in FR2	1727

A.5.5.3.5.1	Test Purpose and Environment	1727

A.5.5.3.5.2	Test Requirements	1730

A.5.5.3.6	Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2	1731

A.5.5.3.6.1	Test Purpose and Environment	1731

A.5.5.3.6.2	Test Requirements	1735

A.5.5.3.7	Direct SCell activation at SCell addition of known SCell in FR2	1736

A.5.5.3.7.1	Test Purpose and Environment	1736

A.5.5.3.7.2	Test Requirements	1739

A.5.5.3.8	Fast SCell Activation of SCell in FR2 intra-band	1739

A.5.5.3.8.1	Test Purpose and Environment	1739

A.5.5.3.8.2	Test Requirements	1743

A.5.5.4	Void	1744

A.5.5.5	Beam Failure Detection and Link recovery procedures	1744

A.5.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1744

A.5.5.5.1.1	Test Purpose and Environment	1744

A.5.5.5.1.2	Test Requirements	1748

A.5.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode	1749

A.5.5.5.2.1	Test Purpose and Environment	1749

A.5.5.5.2.2	Test Requirements	1753

A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1754

A.5.5.5.3.1	Test Purpose and Environment	1754

A.5.5.5.3.2	Test Requirements	1758

A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1759

A.5.5.5.4.1	Test Purpose and Environment	1759

A.5.5.5.4.2	Test Requirements	1763

A.5.5.5.5	EN-DC scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1764

A.5.5.5.5.1	Test Purpose and Environment	1764

A.5.5.5.5.2	Test Requirements	1768

A.5.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1769

A.5.5.5.6.1	Test Purpose and Environment	1769

A.5.5.5.6.2	Test Requirements	1773

A.5.5.5.7	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	1774

A.5.5.5.7.1	Test Purpose and Environment	1774

A.5.5.5.7.2	Test Requirements	1778

A.5.5.5.8	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1779

A.5.5.5.8.1	Test Purpose and Environment	1779

A.5.5.5.8.2	Test Requirements	1784

A.5.5.5.9	Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode for UE fulfilling relaxed measurement criterion	1784

A.5.5.5.9.1	Test Purpose and Environment	1784

A.5.5.5.9.2	Test Requirements	1788

A.5.5.6	Active BWP switch	1788

A.5.5.6.1	DCI-based and Timer-based Active BWP Switch	1788

A.5.5.6.1.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1788

A.5.5.6.1.1.1	Test Purpose and Environment	1788

A.5.5.6.1.1.2	Test Requirements	1792

A.5.5.6.1.2	E-UTRAN – NR PSCell FR2 with FR2 SCell DL active BWP switch in non-DRX in synchronous EN-DC	1792

A.5.5.6.2	RRC-based Active BWP Switch	1797

A.5.5.6.2.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1797

A.5.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1801

A.5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	1801

A.5.5.6.3.1.1	Test Purpose and Environment	1801

A.5.5.6.3.1.2	Test Requirements	1805

A.5.5.6.4	SCell dormancy switch	1805

A.5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time	1805

A.5.5.6.4.1.1	Test Purpose and Environment	1805

A.5.5.6.4.1.2	Test Requirements	1809

A.5.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR2 SCells outside active time	1809

A.5.5.6.4.2.1	Test Purpose and Environment	1809

A.5.5.6.4.2.2	Test Requirements	1816

A.5.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1816

A.5.5.6.5.1	E-UTRAN – NR PSCell FR2  and NR SCell FR2 DL active BWP switch on multiple CCs with non-DRX in synchronous EN-DC	1816

A.5.5.7	PSCell addition and release delay	1819

A.5.5.7.1	Addition and Release Delay of NR PSCell	1819

A.5.5.7.1.1	Test purpose and environment	1819

A.5.5.7.1.2	Test Requirements	1822

A.5.5.8	Active TCI state switch delay	1823

A.5.5.8.1	MAC-CE based active TCI state switch	1823

A.5.5.8.1.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1823

A.5.5.8.1.1.1	Test Purpose and Environment	1823

A.5.5.8.1.1.2	Test Requirements	1826

A.5.5.8.2	RRC based active TCI state switch	1826

A.5.5.8.2.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1826

A.5.5.8.2.1.1	Test Purpose and Environment	1826

A.5.5.8.2.1.2	Test Requirements	1830

A.5.5.9	Uplink spatial relation switch delay	1830

A.5.5.9.1	MAC-CE based uplink spatial relation switch	1830

A.5.5.9.1.1	E-UTRAN – NR PSCell FR2 uplink spatial relation switch for a known spatial relation	1830

A.5.5.9.1.1.1	Test Purpose and Environment	1830

A.5.5.9.1.1.2	Test Requirements	1833

A.5.5.9.2	RRC based spatial relation switch	1833

A.5.5.9.2.1	E-UTRAN – NR PSCell FR2 spatial relation switch associated with a known DL-RS	1833

A.5.5.9.2.1.1	Test Purpose and Environment	1833

A.5.5.9.2.1.2	Test Requirements	1836

A.5.5.10	UE specific CBW change	1836

A.5.5.10.1	UE specific CBW change on FR2 NR PSCell	1836

A.5.5.10.1.1	Test Purpose and Environment	1836

A.5.5.10.1.2	Test Requirements	1839

A.5.5.11	Unified TCI state switch delay	1840

A.5.5.11.1	MAC-CE based active joint TCI state switch	1840

A.5.5.11.1.1	E-UTRAN – NR PSCell FR2 active joint TCI state switch for a known TCI state	1840

A.5.5.11.1.1.1	Test Purpose and Environment	1840

A.5.5.11.1.1.2	Test parameters	1840

A.5.5.11.1.1.3	Test Requirements	1843

A.5.5.11.2	MAC-CE based active uplink TCI state switch	1843

A.5.5.11.2.1	E-UTRAN – NR PSCell FR2 active uplink TCI state switch for a known TCI state	1843

A.5.5.11.2.1.1	Test Purpose and Environment	1843

A.5.5.11.2.1.2	Test parameters	1844

A.5.5.11.2.1.3	Test Requirements	1846

A.5.5.11.3	MAC-CE based active downlink TCI state switch	1846

A.5.5.11.3.1	E-UTRAN – NR PSCell FR2 downlink TCI state switch to cell with additional PCI for a known TCI state	1846

A.5.5.11.3.1.1	Test Purpose and Environment	1846

A.5.5.11.3.1.2	Test Parameters	1847

A.5.5.11.3.1.3	Test Requirements	1851

A.5.5.12	PSCell activation and deactivation delay	1851

A.5.5.12.1	PSCell activation and deactivation delay	1851

A.5.5.12.1.1	Test purpose and environment	1851

A.5.5.12.1.2	Test Requirements	1854

A.5.5.13	Conditional PSCell addition and release delay	1855

A.5.5.13.1	Addition and Release Delay of NR PSCell	1855

A.5.5.13.1.1	Test purpose and environment	1855

A.5.5.13.1.2	Test Requirements	1858

A.5.6	Measurement procedure	1859

A.5.6.1	Intra-frequency Measurements	1859

A.5.6.1.1	EN-DC event triggered reporting test without gap under non-DRX	1859

A.5.6.1.1.1	Test purpose and Environment	1859

A.5.6.1.1.2	Test Requirements	1862

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX	1862

A.5.6.1.2.1	Test purpose and Environment	1862

A.5.6.1.2.2	Test Requirements	1864

A.5.6.1.3	EN-DC event triggered reporting test with per-UE gaps under non-DRX	1865

A.5.6.1.3.1	Test purpose and Environment	1865

A.5.6.1.3.2	Test Requirements	1869

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX	1869

A.5.6.1.4.1	Test purpose and Environment	1869

A.5.6.1.4.2	Test Requirements	1872

A.5.6.2	Inter-frequency Measurements	1873

A.5.6.2.1 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1873

A.5.6.2.1.1	Test Purpose and Environment	1873

A.5.6.2.1.2	Test Requirements	1876

A.5.6.2.2 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1876

A.5.6.2.2.1	Test Purpose and Environment	1876

A.5.6.2.2.2	Test Requirements	1880

A.5.6.2.3 	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1880

A.5.6.2.3.1	Test Purpose and Environment	1880

A.5.6.2.3.2	Test Requirements	1885

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1885

A.5.6.2.4.1	Test Purpose and Environment	1885

A.5.6.2.4.2	Test Requirements	1889

A.5.6.2.5	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1889

A.5.6.2.5.1	Test Purpose and Environment	1889

A.5.6.2.5.2	Test Requirements	1894

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1894

A.5.6.2.6.1	Test Purpose and Environment	1894

A.5.6.2.6.2	Test Requirements	1898

A.5.6.2.7	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1898

A.5.6.2.7.1	Test Purpose and Environment	1898

A.5.6.2.7.2	Test Requirements	1903

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1903

A.5.6.2.8.1	Test Purpose and Environment	1903

A.5.6.2.8.2	Test Requirements	1908

A.5.6.3	L1-RSRP measurement for beam reporting	1909

A.5.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1909

A.5.6.3.1.1	Test Purpose and Environment	1909

A.5.6.3.1.2	Test parameters	1909

A.5.6.3.1.3	Test Requirements	1911

A.5.6.3.2	SSB based L1-RSRP measurement when DRX is used	1911

A.5.6.3.2.1	Test Purpose and Environment	1911

A.5.6.3.2.2	Test parameters	1912

A.5.6.3.2.3	Test Requirements	1914

A.5.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	1914

A.5.6.3.3.1	Test Purpose and Environment	1914

A.5.6.3.3.2	Test parameters	1915

A.5.6.3.3.3	Test Requirements	1917

A.5.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	1917

A.5.6.3.4.1	Test Purpose and Environment	1917

A.5.6.3.4.2	Test parameters	1918

A.5.6.3.4.3	Test Requirements	1920

A.5.6.4	CLI measurements	1921

A.5.6.4.1	SRS-RSRP measurement with DRX	1921

A.5.6.4.1.1	Test Purpose and Environment	1921

A.5.6.4.1.2	Test Parameters	1921

A.5.6.4.1.3	Test Requirements	1923

A.5.6.4.2	CLI-RSSI measurement with DRX	1923

A.5.6.4.2.1	Test Purpose and Environment	1923

A.5.6.4.2.2	Test Parameters	1924

A.5.6.4.2.3	Test Requirements	1926

A.5.6.5	Measurements with autonomous gaps	1926

A.5.6.5.1 	EN-DC inter-frequency CGI identification of NR neighbor cell in FR2	1926

A.5.6.5.1.1	Test Purpose and Environment	1926

A.5.6.5.1.2	Test Requirements	1929

A.5.6.6	L1-SINR measurement for beam reporting	1930

A.5.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	1932

A.5.6.6.2.1	Test Purpose and Environment	1932

A.5.6.6.2.2	Test parameters	1933

A.5.6.6.2.3	Test Requirements	1936

A.5.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	1936

A.5.6.6.3.1	Test Purpose and Environment	1936

A.5.6.6.3.2	Test parameters	1937

A.5.6.6.3.3	Test Requirements	1939

A.5.6.7	CSI-RS based Intra-frequency Measurements	1939

A.5.6.7.1	EN-DC event triggered reporting test without gap under non-DRX	1939

A.5.6.7.1.1	Test purpose and Environment	1939

A.5.6.7.1.2	Test Requirements	1942

A.5.6.8	CSI-RS based Inter-frequency Measurements	1942

A.5.6.8.1 	EN-DC event triggered reporting tests for NR FR2 cell when DRX is used	1942

A.5.6.8.1.1	Test Purpose and Environment	1942

A.5.6.8.1.2	Test Requirements	1947

A.5.7	Measurement Performance requirements	1947

A.5.7.1	SS-RSRP	1947

A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1947

A.5.7.1.1.1	Test Purpose and Environment	1947

A.5.7.1.1.2	Test parameters	1948

A.5.7.1.1.3	Test Requirements	1950

A.5.7.1.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1951

A.5.7.1.2.1	Test Purpose and Environment	1951

A.5.7.1.2.2	Test parameters	1951

A.5.7.1.2.3	Test Requirements	1955

A.5.7.1.3	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	1956

A.5.7.1.3.1	Test Purpose and Environment	1956

A.5.7.1.3.2	Test parameters	1956

A.5.7.1.3.3	Test Requirements	1959

A.5.7.2	SS-RSRQ	1960

A.5.7.2.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1960

A.5.7.2.1.1	Test Purpose and Environment	1960

A.5.7.2.1.2	Test Parameters	1960

A.5.7.2.1.3	Test Requirements	1963

A.5.7.2.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1963

A.5.7.2.2.1	Test Purpose and Environment	1963

A.5.7.2.2.2	Test Parameters	1963

A.5.7.2.2.3	Test Requirements	1965

A.5.7.3	SS-SINR	1966

A.5.7.3.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1966

A.5.7.3.1.1	Test Purpose and Environment	1966

A.5.7.3.1.2	Test Parameters	1966

A.5.7.3.1.3	Test Requirements	1969

A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1969

A.5.7.3.2.1	Test Purpose and Environment	1969

A.5.7.3.2.2	Test Parameters	1969

A.5.7.3.2.3	Test Requirements	1971

A.5.7.4	L1-RSRP measurement for beam reporting	1971

A.5.7.4.1	SSB based L1-RSRP measurement	1971

A.5.7.4.1.1	Test Purpose and Environment	1971

A.5.7.4.1.2	Test parameters	1972

A.5.7.4.1.3	Test Requirements	1974

A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1975

A.5.7.4.2.1	Test Purpose and Environment	1975

A.5.7.4.2.2	Test parameters	1975

A.5.7.4.2.3	Test Requirements	1977

A.5.7.5	CLI measurements	1978

A.5.7.5.1	EN-DC SRS-RSRP measurement accuracy with FR2 serving cell	1978

A.5.7.5.1.1	Test Purpose and Environment	1978

A.5.7.5.1.2	Test parameters	1978

A.5.7.5.1.3	Test Requirements	1981

A.5.7.5.2	EN-DC CLI-RSSI measurement accuracy with FR2 serving cell	1982

A.5.7.5.2.1	Test Purpose and Environment	1982

A.5.7.5.2.2	Test parameters	1982

A.5.7.5.2.3	Test Requirements	1984

A.5.7.6	L1-SINR measurement for beam reporting	1985

A.5.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1988

A.5.7.6.2.1	Test Purpose and Environment	1988

A.5.7.6.2.2	Test parameters	1988

A.5.7.6.2.3	Test Requirements	1990

A.5.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1991

A.5.7.6.3.1	Test Purpose and Environment	1991

A.5.7.6.3.2	Test parameters	1991

A.5.7.6.3.3	Test Requirements	1993

A.5.7.7	CSI-RSRP	1994

A.5.7.7.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1994

A.5.7.7.1.1	Test Purpose and Environment	1994

A.5.7.7.1.2	Test parameters	1994

A.5.7.7.1.3	Test Requirements	1996

A.5.7.7.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1997

A.5.7.7.2.1	Test Purpose and Environment	1997

A.5.7.7.2.2	Test parameters	1997

A.5.7.7.2.3	Test Requirements	2001

A.5.7.8	CSI-RSRQ	2002

A.5.7.8.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2002

A.5.7.8.1.1	Test Purpose and Environment	2002

A.5.7.8.1.2	Test Parameters	2002

A.5.7.8.1.3	Test Requirements	2004

A.5.7.8.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2004

A.5.7.8.2.1	Test Purpose and Environment	2004

A.5.7.8.2.2	Test Parameters	2004

A.5.7.8.2.3	Test Requirements	2006

A.5.7.9	CSI-SINR	2006

A.5.7.9.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2006

A.5.7.9.1.1	Test Purpose and Environment	2006

A.5.7.9.1.2	Test Parameters	2006

A.5.7.9.1.3	Test Requirements	2009

A.5.7.9.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2009

A.5.7.9.2.1	Test Purpose and Environment	2009

A.5.7.9.2.2	Test Parameters	2009

A.5.7.9.2.3	Test Requirements	2012

A.5.8	Void	2012

A.6	NR standalone tests with all NR cells in FR1	2005

A.6.1	SA: RRC_IDLE state mobility	2005

A.6.1.1	Cell re-selection to NR	2005

A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case	2005

A.6.1.1.1.1	Test Purpose and Environment	2005

A.6.1.1.1.2	Test Parameters	2005

A.6.1.1.1.3	Test Requirements	2009

A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case	2009

A.6.1.1.2.1	Test Purpose and Environment	2009

A.6.1.1.2.2	Test Parameters	2009

A.6.1.1.2.3	Test Requirements	2013

A.6.1.1.3	Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2013

A.6.1.1.3.1	Test Purpose and Environment	2013

A.6.1.1.3.2	Test Parameters	2013

A.6.1.1.3.3	Test Requirements	2018

A.6.1.1.4	Cell reselection to FR1 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2018

A.6.1.1.4.1	Test Purpose and Environment	2018

A.6.1.1.4.2	Test Parameters	2018

A.6.1.1.4.3	Test Requirements	2021

A.6.1.1.5	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2021

A.6.1.1.5.1	Test Purpose and Environment	2021

A.6.1.1.5.2	Test Parameters	2021

A.6.1.1.5.3	Test Requirements	2026

A.6.1.1.6	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2027

A.6.1.1.6.1	Test Purpose and Environment	2027

A.6.1.1.6.2	Test Parameters	2027

A.6.1.1.6.3	Test Requirements	2031

A.6.1.1.7	Cell reselection to FR1 intra-frequency NR case for UE configured with highSpeedMeasFlag-r16	2032

A.6.1.1.7.1	Test Purpose and Environment	2032

A.6.1.1.7.2	Test Parameters	2032

A.6.1.1.7.3	Test Requirements	2035

A.6.1.1.8	Cell reselection to FR1 inter-frequency NR case for UE configured with highSpeedMeasInterFreq-r17	2035

A.6.1.1.8.1	Test Purpose and Environment	2035

A.6.1.1.8.2	Test Parameters	2035

A.6.1.1.8.3	Test Requirements	2039

A.6.1.2	Inter-RAT E-UTRAN cell re-selection	2039

A.6.1.2.1	Cell reselection to higher priority E-UTRAN	2039

A.6.1.2.1.1	Test Purpose and Environment	2039

A.6.1.2.1.2	Test Parameters	2039

A.6.1.2.1.3	Test Requirements	2042

A.6.1.2.2	Cell reselection to lower priority E-UTRAN	2043

A.6.1.2.2.1	Test Purpose and Environment	2043

A.6.1.2.2.2	Test Parameters	2043

A.6.1.2.2.3	Test Requirements	2046

A.6.1.2.3	Cell reselection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	2047

A.6.1.2.3.1	Test Purpose and Environment	2047

A.6.1.2.3.2	Test Parameters	2047

A.6.1.2.3.3	Test Requirements	2050

A.6.1.2.4	Cell reselection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	2051

A.6.1.2.4.1	Test Purpose and Environment	2051

A.6.1.2.4.2	Test Parameters	2051

A.6.1.2.4.3	Test Requirements	2054

A.6.1.2.5	Cell reselection to lower priority E-UTRAN cell for UE configured with highSpeedMeasFlag-r16	2055

A.6.1.2.5.1	Test Purpose and Environment	2055

A.6.1.2.5.2	Test Parameters	2055

A.6.1.2.5.3	Test Requirements	2059

A.6.1.1.7	Void	2060

A.6.2	SA: RRC_INACTIVE state mobility	2060

A.6.2.1	Configured Grant based Small Data Transmissions (CG-SDT)	2060

A.6.2.1.1	Test purpose and Environment	2060

A.6.2.1.2	Test Parameters	2061

A.6.2.1.3	Test requirements	2063

A.6.3	RRC_CONNECTED state mobility	2063

A.6.3.1	Handover	2063

A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell	2063

A.6.3.1.1.1	Test Purpose and Environment	2063

A.6.3.1.1.2	Test Parameters	2063

A.6.3.1.1.3 Test Requirements	2067

A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell	2067

A.6.3.1.2.1	Test Purpose and Environment	2067

A.6.3.1.2.2	Test Parameters	2067

A.6.3.1.2.3	Test Requirements	2071

A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell	2071

A.6.3.1.3.1	Test Purpose and Environment	2071

A.6.3.1.3.2	Test Parameters	2071

A.6.3.1.3.3	Test Requirements	2075

A.6.3.1.4	SA NR - E-UTRAN handover	2075

A.6.3.1.4.1	Test Purpose and Environment	2075

A.6.3.1.4.2	Test Requirements	2081

A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell	2081

A.6.3.1.5.1	Test Purpose and Environment	2081

A.6.3.1.5.2	Test Requirements	2087

A.6.3.1.6	 SA NR - UTRAN FDD handover	2087

A.6.3.1.6.1	Test Purpose and Environment	2087

A.6.3.1.6.2	Test Requirements	2091

A.6.3.1.7	Intra-frequency synchronous DAPS handover in FR1	2091

A.6.3.1.7.1	Test Purpose and Environment	2091

A.6.3.1.7.2	Test Parameters	2091

A.6.3.1.7.3	Test Requirements	2095

A.6.3.1.8	Intra-frequency asynchronous DAPS handover in FR1	2096

A.6.3.1.8.1	Test Purpose and Environment	2096

A.6.3.1.8.2	Test Parameters	2096

A.6.3.1.8.3	Test Requirements	2100

A.6.3.1.9	Intra-band inter-frequency synchronous DAPS handover test in SA for FR1	2101

A.6.3.1.9.1	Test Purpose and Environment	2101

A.6.3.1.9.2	Test Parameters	2101

A.6.3.1.9.3	Test Requirements	2105

A.6.3.1.10	Intra-band inter-frequency asynchronous DAPS handover test in SA for FR1	2105

A.6.3.1.10.1	Test Purpose and Environment	2105

A.6.3.1.10.2	Test Parameters	2105

A.6.3.1.10.3	Test Requirements	2108

A.6.3.1.11	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1	2108

A.6.3.1.11.1	Test Purpose and Environment	2108

A.6.3.1.11.2	Test Parameters	2108

A.6.3.1.11.3 Test Requirements	2115

A.6.3.1.12	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1	2115

A.6.3.1.12.1	Test Purpose and Environment	2115

A.6.3.1.12.2	Test Parameters	2115

A.6.3.1.12.3 Test Requirements	2123

A.6.3.1.13	SA NR - E-UTRAN with NR PSCell addition in FR1	2123

A.6.3.1.13.1	Test Purpose and Environment	2123

A.6.3.1.13.2	Test Requirements	2132

A.6.3.1.14	SA NR - E-UTRAN handover with NR FR1 PScell addition	2132

A.6.3.1.14.1	Test Purpose and Environment	2132

A.6.3.1.14.2	Test Requirements	2141

A.6.3.2	RRC Connection Mobility Control	2142

A.6.3.2.1	SA: RRC Re-establishment	2142

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1	2142

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1	2146

A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing	2150

A.6.3.2.2	Random Access	2153

A.6.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	2153

A.6.3.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	2157

A.6.3.2.2.3	2-step RA type contention based random access test in FR1 for NR standalone	2162

A.6.3.2.2.4	2-step RA type non-contention based test in FR1 for NR standalone	2167

A.6.3.2.3	SA: RRC Connection Release with Redirection	2171

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1	2171

A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN	2175

A.6.3.3 Conditional handover	2182

A.6.3.3.1	Intra-frequency conditional handover from FR1 to FR1	2182

A.6.3.3.1.1	Test Purpose and Environment	2182

A.6.3.3.1.2	Test Parameters	2182

A.6.3.3.1.3 Test Requirements	2186

A.6.3.3.2	Inter-frequency conditional handover from FR1 to FR1	2186

A.6.3.3.2.1	Test Purpose and Environment	2186

A.6.3.3.2.2	Test Parameters	2186

A.6.3.3.2.3 Test Requirements	2190

A.6.4	Timing	2190

A.6.4.1	UE transmit timing	2190

A.6.4.1.1	NR UE Transmit Timing Test for FR1	2190

A.6.4.1.1.1	Test Purpose and environment	2190

A.6.4.1.1.2	Test requirements	2194

A.6.4.2	UE timer accuracy	2194

A.6.4.3	Timing advance	2194

A.6.4.3.1	SA FR1 timing advance adjustment accuracy	2194

A.6.4.3.1.1	Test Purpose and Environment	2194

A.6.4.3.1.2	Test Parameters	2194

A.6.4.3.1.3	Test Requirements	2198

A.6.5	Signalling characteristics	2198

A.6.5.1	Radio link Monitoring	2198

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	2199

A.6.5.1.1.1	Test Purpose and Environment	2199

A.6.5.1.1.2	Test Requirements	2204

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	2204

A.6.5.1.2.1	Test Purpose and Environment	2204

A.6.5.1.2.2	Test Requirements	2210

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	2210

A.6.5.1.3.1	Test Purpose and Environment	2210

A.6.5.1.3.2	Test Requirements	2216

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	2216

A.6.5.1.4.1	Test Purpose and Environment	2216

A.6.5.1.4.2	Test Requirements	2222

A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	2222

A.6.5.1.5.1	Test Purpose and Environment	2222

A.6.5.1.5.2	Test Requirements	2228

A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	2228

A.6.5.1.6.1	Test Purpose and Environment	2228

A.6.5.1.6.2	Test Requirements	2233

A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	2233

A.6.5.1.7.1	Test Purpose and Environment	2233

A.6.5.1.7.2	Test Requirements	2238

A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	2238

A.6.5.1.8.1	Test Purpose and Environment	2238

A.6.5.1.8.2	Test Requirements	2243

A.6.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion	2243

A.6.5.1.9.1	Test Purpose and Environment	2243

A.6.5.1.9.2	Test Requirements	2249

A.6.5.2	Interruption	2249

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1	2249

A.6.5.2.1.2	Test Requirements	2253

A.6.5.2.2	SA interruptions at NR SRS carrier based switching	2254

A.6.5.2.2.1	Test Purpose and Environment	2254

A.6.5.2.2.2	Test Parameters	2254

A.6.5.2.2.3	Test Requirements	2256

A.6.5.2.3	SA interruptions at NR SRS antenna port switching with 1 SRS symbol in a slot in NR-CA	2257

A.6.5.2.3.1	Test Purpose and Environment	2257

A.6.5.2.3.2	Test Parameters	2257

A.6.5.2.3.3	Test Requirements	2259

A.6.5.2.4	SA interruptions at NR SRS antenna port switching	2260

A.6.5.2.4.1	Test Purpose and Environment	2260

A.6.5.2.4.2	Test Parameters	2260

A.6.5.2.4.3	Test Requirements	2262

A.6.5.3	SCell Activation and Deactivation Delay	2263

A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	2263

A.6.5.3.1.1	Test Purpose and Environment	2263

A.6.5.3.1.2	Test Requirements	2268

A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	2269

A.6.5.3.2.1	Test Purpose and Environment	2269

A.6.5.3.2.2	Test Requirements	2269

A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	2269

A.6.5.3.3.1	Test Purpose and Environment	2269

A.6.5.3.3.2	Test Requirements	2270

A.6.5.3.4	Direct SCell activation at SCell addition of known SCell in FR1	2270

A.6.5.3.4.1	Test Purpose and Environment	2270

A.6.5.3.4.2	Test Requirements	2275

A.6.5.3.5	Direct SCell activation at handover with known SCell in FR1	2275

A.6.5.3.5.1	Test Purpose and Environment	2275

A.6.5.3.5.2	Test Requirements	2280

A.6.5.3.4	Fast SCell Activation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	2280

A.6.5.3.4.1	Test Purpose and Environment	2280

A.6.5.3.4.2	Test Requirements	2284

A.6.5.3.5	SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	2284

A.6.5.3.5.1	Test Purpose and Environment	2284

A.6.5.3.5.2	Test Requirements	2285

A.6.5.3.6	PUCCH SCell Activation and deactivation of known SCell in FR1	2285

A.6.5.3.6.1	Test Purpose and Environment	2285

A.6.5.3.6.2	Test Requirements	2289

A.6.5.3.7	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	2289

A.6.5.3.7.1	Test Purpose and Environment	2289

A.6.5.3.7.2	Test Requirements	2293

A.6.5.3.8	SCell Activation and Deactivation of one FR1 known PUCCH SCell and one FR1 unknown SCell with single activation/deactivation command	2294

A.6.5.3.8.1	Test Purpose and Environment	2294

A.6.5.3.8.2	Test Requirements	2298

A.6.5.3.9	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR1 in non-DRX	2299

A.6.5.3.9.1	Test Purpose and Environment	2299

A.6.5.3.9.2	Test Requirements	2303

A.6.5.4	UE UL carrier RRC reconfiguration Delay	2304

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay	2304

A.6.5.4.1.1	Test Purpose and Environment	2304

A.6.5.4.1.2	Test Requirements	2314

A.6.5.4.2	Void	2315

A.6.5.5	Beam Failure Detection and Link recovery procedures	2315

A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	2315

A.6.5.5.1.1	Test Purpose and Environment	2315

A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	2320

A.6.5.5.2.1	Test Purpose and Environment	2320

A.6.5.5.2.2	Test Requirements	2327

A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	2327

A.6.5.5.3.1	Test Purpose and Environment	2327

A.6.5.5.3.2	Test Requirements	2333

A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	2333

A.6.5.5.4.1	Test Purpose and Environment	2333

A.6.5.5.4.2	Test Requirements	2339

A.6.5.5.5	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	2339

A.6.5.5.5.1	Test Purpose and Environment	2339

A.6.5.5.5.2	Test Requirements	2343

A.6.5.5.6	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	2344

A.6.5.5.6.1	Test Purpose and Environment	2344

A.6.5.5.6.2	Test Requirements	2348

A.6.5.5.7	TRP Specific Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	2348

A.6.5.5.7.1	Test Purpose and Environment	2348

A.6.5.5.7.2	Test Requirements	2354

A.6.5.6	Active BWP switch	2354

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch	2354

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of SCell with non-DRX in SA	2354

A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA	2360

A.6.5.6.2	RRC-based Active BWP Switch	2365

A.6.5.6.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	2365

A.6.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	2369

A.6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2369

A.6.5.6.3.1.1	Test Purpose and Environment	2369

A.6.5.6.3.1.2	Test Requirements	2376

A.6.5.6.4	SCell dormancy switch	2376

A.6.5.6.4.1	NR FR1 PCell SCell dormancy switch of single FR1 SCell outside active time	2376

A.6.5.6.4.1.1	Test Purpose and Environment	2376

A.6.5.6.4.1.2	Test Requirements	2382

A.6.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR1 SCells inside active time	2382

A.6.5.6.4.2.1	 Test Purpose and Environment	2382

A.6.5.6.4.2.2	Test Requirements	2389

A.6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	2389

A.6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2389

A.6.5.7	DL interruptions at switching between two uplink carriers	2394

A.6.5.7.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	2394

A.6.5.7.1.1	Test Purpose and Environment	2394

A.6.5.7.1.2	Test Requirements	2398

A.6.5.7.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	2398

A.6.5.7.2.1	Test Purpose and Environment	2398

A.6.5.7.2.2	Test Requirements	2402

A.6.5.7A	DL interruptions at switching between two uplink carriers with two transmit antenna connectors	2402

A.6.5.7A.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	2402

A.6.5.7A.1.1	Test Purpose and Environment	2402

A.6.5.7A.1.2	Test Requirements	2406

A.6.5.7A.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	2406

A.6.5.7A.2.1	Test Purpose and Environment	2406

A.6.5.7A.2.2	Test Requirements	2410

A.6.5.7B	DL interruptions at switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	2410

A.6.5.7B.1	DL interruptions at switching between two uplink bands in FDD-TDD CA	2410

A.6.5.7B.1.1	Test Purpose and Environment	2410

A.6.5.7B.1.2	Test Requirements	2414

A.6.5.7B.2	DL interruptions at switching between two uplink bands in TDD-TDD CA	2414

A.6.5.7B.2.1	Test Purpose and Environment	2414

A.6.5.7B.2.2	Test Requirements	2418

A.6.5.7C	DL interruptions at switching between two uplink bands with two transmit antenna connectors	2418

A.6.5.7C.1	DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA	2418

A.6.5.7C.1.1	Test Purpose and Environment	2418

A.6.5.7C.1.2	Test Requirements	2422

A.6.5.7C.2	DL interruptions at switching between two uplink bands with two transmit antenna connectors in TDD-TDD CA	2423

A.6.5.7C.2.1	Test Purpose and Environment	2423

A.6.5.7C.2.2	Test Requirements	2427

A.6.5.8	UE specific CBW change	2428

A.6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	2428

A.6.5.8.1.1	Test Purpose and Environment	2428

A.6.5.8.1.2	Test Requirements	2432

A.6.5.9	Pathloss reference signal switching delay	2432

A.6.5.9.1	MAC-CE based pathloss reference signal switch delay	2432

A.6.5.9.1.1	Test Purpose and Environment	2432

A.6.5.9.1.2	Test Requirements	2435

A.6.6	Measurement procedure	2436

A.6.6.1	Intra-frequency Measurements	2436

A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX	2436

A.6.6.1.1.1	Test purpose and Environment	2436

A.6.6.1.1.2	Test parameters	2436

A.6.6.1.1.3	Test Requirements	2440

A.6.6.1.2	SA event triggered reporting tests without gap under DRX	2440

A.6.6.1.2.1	Test purpose and Environment	2440

A.6.6.1.2.2	Test parameters	2440

A.6.6.1.2.3	Test Requirements	2444

A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX	2444

A.6.6.1.3.1	Test purpose and Environment	2444

A.6.6.1.3.2	Test parameters	2444

A.6.6.1.3.3	Test Requirements	2448

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX	2448

A.6.6.1.4.1	Test purpose and Environment	2448

A.6.6.1.4.2	Test parameters	2448

A.6.6.1.4.3	Test Requirements	2452

A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading	2452

A.6.6.1.5.1	Test purpose and Environment	2452

A.6.6.1.5.2	Test parameters	2452

A.6.6.1.5.3	Test Requirements	2454

A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	2455

A.6.6.1.6.1	Test purpose and Environment	2455

A.6.6.1.6.2	Test parameters	2455

A.6.6.1.6.3	Test Requirements	2456

A.6.6.1.7	SA event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	2457

A.6.6.1.7.1	Test purpose and Environment	2457

A.6.6.1.7.2	Test parameters	2457

A.6.6.1.7.3	Test Requirements	2461

A.6.6.1.8	SA event triggered reporting tests without gap under DRX for UE configured with highSpeedMeasCA-Scell-r17	2461

A.6.6.1.8.1	Test purpose and Environment	2461

A.6.6.1.8.2	Test parameters	2461

A.6.6.1.8.3	Test Requirements	2465

A.6.6.2	Inter-frequency Measurements	2465

A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2465

A.6.6.2.1.1	Test Purpose and Environment	2465

A.6.6.2.1.2	Test Requirements	2469

A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2469

A.6.6.2.2.1	Test Purpose and Environment	2469

A.6.6.2.2.2	Test Requirements	2473

A.6.6.2.3	Void	2474

A.6.6.2.4	Void	2474

A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2474

A.6.6.2.5.1	Test Purpose and Environment	2474

A.6.6.2.5.2	Test Requirements	2478

A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2478

A.6.6.2.6.1	Test Purpose and Environment	2478

A.6.6.2.6.2	Test Requirements	2482

A.6.6.2.7	Void	2483

A.6.6.2.8	Void	2483

A.6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern	2483

A.6.6.2.9.1	Test Purpose and Environment	2483

A.6.6.2.9.2	Test Requirements	2486

A.6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used	2486

A.6.6.2.10.1	Test Purpose and Environment	2486

A.6.6.2.10.2	Test Requirements	2490

A.6.6.2.12	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for UE configured with highSpeedMeasInterFreq-r17	2494

A.6.6.2.12.1	Test Purpose and Environment	2494

A.6.6.2.12.2	Test Requirements	2498

A.6.6.3	Inter-RAT Measurements	2498

A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	2498

A.6.6.3.1.1	Test Purpose and Environment	2498

A.6.6.3.1.2	Test Requirements	2504

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	2504

A.6.6.3.2.1	Test Purpose and Environment	2504

A.6.6.3.2.2	Test Requirements	2511

A.6.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for UE configured with highSpeedMeasFlag-r16	2511

A.6.6.3.3.1	Test Purpose and Environment	2511

A.6.6.3.3.2	Test Requirements	2516

A.6.6.4	L1-RSRP measurement for beam reporting	2516

A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used	2516

A.6.6.4.1.1	Test Purpose and Environment	2516

A.6.6.4.1.2	Test parameters	2517

A.6.6.4.1.3	Test Requirements	2520

A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used	2520

A.6.6.4.2.1	Test Purpose and Environment	2520

A.6.6.4.2.2	Test parameters	2521

A.6.6.4.2.3	Test Requirements	2524

A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	2524

A.6.6.4.3.1	Test Purpose and Environment	2524

A.6.6.4.3.2	Test parameters	2525

A.6.6.4.3.3	Test Requirements	2528

A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	2528

A.6.6.4.4.1	Test Purpose and Environment	2528

A.6.6.4.4.2	Test parameters	2529

A.6.6.4.4.3	Test Requirements	2532

A.6.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	2532

A.6.6.4.5.1	Test Purpose and Environment	2532

A.6.6.4.5.2	Test parameters	2533

A.6.6.4.5.3	Test Requirements	2536

A.6.6.4.6	Inter-cell SSB based L1-RSRP measurements on FR1 PCell when DRX is used	2536

A.6.6.4.6.1	Test Purpose and Environment	2536

A.6.6.4.6.2	Test parameters	2537

A.6.6.4.6.3	Test Requirements	2540

A.6.6.5	Inter-RAT UTRAN FDD measurements	2541

A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1	2541

A.6.6.5.1.1	Test Purpose and Environment	2541

A.6.6.5.1.2	Test Requirements	2544

A.6.6.6	CLI measurements	2544

A.6.6.6.1	SRS-RSRP measurement with DRX	2544

A.6.6.6.1.1	Test Purpose and Environment	2544

A.6.6.6.1.2	Test Parameters	2545

A.6.6.6.1.3	Test Requirements	2548

A.6.6.6.2	CLI-RSSI measurement with DRX	2548

A.6.6.6.2.1	Test Purpose and Environment	2548

A.6.6.6.2.2	Test Parameters	2549

A.6.6.6.2.3	Test Requirements	2551

A.6.6.7	NR measurements with autonomous gaps	2551

A.6.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	2551

A.6.6.7.1.1	Test Purpose and Environment	2551

A.6.6.7.1.2	Test Parameters	2551

A.6.6.7.1.3	Test Requirements	2554

A.6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	2554

A.6.6.7.2.1	Test Purpose and Environment	2554

A.6.6.7.2.2	Test Requirements	2557

A.6.6.8	L1-SINR measurement for beam reporting	2558

A.6.6.8.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	2560

A.6.6.8.2.1	Test Purpose and Environment	2560

A.6.6.8.2.2	Test parameters	2560

A.6.6.8.2.3	Test Requirements	2564

A.6.6.8.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	2564

A.6.6.8.3.1	Test Purpose and Environment	2564

A.6.6.8.3.2	Test parameters	2565

A.6.6.8.3.3	Test Requirements	2567

A.6.6.9	Idle Mode CA/DC Measurements	2567

A.6.6.9.1	SA Idle mode CA/DC measurement for FR1	2567

A.6.6.9.1.1	Test Purpose and Environment	2567

A.6.6.9.1.2	Test Requirements	2574

A.6.6.10	CSI-RS based intra-frequency Measurements	2574

A.6.6.10.1	SA event triggered reporting tests without gap under non-DRX	2574

A.6.6.10.1.1	Test purpose and Environment	2574

A.6.6.10.1.2	Test Requirements	2577

A.6.6.11	CSI-RS based inter-frequency Measurements	2577

A.6.6.11.1	 SA event triggered reporting tests with gap under DRX	2577

A.6.6.11.1.1	Test Purpose and Environment	2577

A.6.6.11.1.2	Test Requirements	2581

A.6.6.12	RSTD measurements	2581

A.6.6.12.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA	2581

A.6.6.12.1.1	Test Purpose and Environment	2581

A.6.6.12.1.2	Test Requirements	2588

A.6.6.12.2	NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR1 SA	2589

A.6.6.12.2.1	Test Purpose and Environment	2589

A.6.6.12.2.2	Test Requirements	2595

A.6.6.12.3	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA	2595

A.6.6.12.3.1	Test Purpose and Environment	2595

A.6.6.12.3.2	Test Requirements	2601

A.6.6.12.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA without measurement gap	2601

A.6.6.12.4.1	Test Purpose and Environment	2601

A.6.6.12.4.2	Test Requirements	2605

A.6.6.12.5	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_CONNECTED state with Rx TEG	2606

A.6.6.12.5.1	Test Purpose and Environment	2606

A.6.6.12.5.2	Test Requirements	2610

A.6.6.13 PRS-RSRP measurements	2610

A.6.6.13.1	PRS-RSRP reporting delay test case for single positioning frequency layer	2610

A.6.6.13.1.1	Test purpose and Environment	2610

A.6.6.13.1.2	Test Requirements	2614

A.6.6.13.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	2614

A.6.6.13.2.1	Test purpose and Environment	2614

A.6.6.13.2.2	Test Requirements	2618

A.6.6.13.3	PRS-RSRP reporting delay test case for reduced number of samples	2618

A.6.6.13.3.1	Test purpose and Environment	2618

A.6.6.13.3.2	Test Requirements	2622

A.6.6.13.4	PRS-RSRP reporting delay test case for single positioning frequency layer outside MG	2622

A.6.6.13.4.1	Test purpose and Environment	2622

A.6.6.14	UE Rx-Tx time difference measurements	2626

A.6.6.14.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	2626

A.6.6.14.1.1	Test purpose and environment	2626

A.6.6.14.1.2	Test requirements	2630

A.6.6.14.2	UE Rx-Tx time difference measurement for dual positioning frequency layers in FR1 SA	2630

A.6.6.14.2.1	Test purpose and environment	2630

A.6.6.14.2.2	Test requirements	2634

A.6.6.14.3	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with reduced sample number	2634

A.6.6.14.3.1	Test purpose and environment	2634

A.6.6.14.3.2	Test requirements	2638

A.6.6.14.4	UE Rx-Tx time difference measurement without gaps in FR1 SA	2638

A.6.6.14.4.1	Test purpose and environment	2638

A.6.6.14.4.2	Test requirements	2642

A.6.6.14.5	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with multiple RxTx TEGs	2642

A.6.6.14.4.1	Test purpose and environment	2642

A.6.6.14.4.2	Test requirements	2646

A.6.6.15	Idle Mode measurements of inter-RAT DC candidate cells for early reporting	2646

A.6.6.15.1	Test Purpose and Environment	2646

A.6.6.15.2	Test Requirements	2653

A.6.6.16	PRS-RSRPP measurements	2654

A.6.6.16.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state	2654

A.6.6.16.1.1	Test purpose and Environment	2654

A.6.6.16.1.2	Test Requirements	2656

A.6.6.16.2	PRS-RSRPP reporting delay test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state	2656

A.6.6.16.2.1	Test purpose and Environment	2656

A.6.6.16.2.2	Test Requirements	2659

A.6.6.16.3	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state without measurement gap	2659

A.6.6.16.3.1	Test purpose and Environment	2659

A.6.6.16.3.2	Test Requirements	2663

A.6.6.17	SA event triggered reporting tests with Pre-MG	2663

A.6.6.17.1	SA event triggered reporting tests with autonomous activation/deactivation Pre-MG	2663

A.6.6.17.1.1	Test purpose and Environment	2663

A.6.6.17.1.2	Test parameters	2663

A.6.6.17.1.3	Test Requirements	2667

A.6.6.17.2	SA event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation	2667

A.6.6.17.2.1	Test purpose and Environment	2667

A.6.6.17.2.2	Test parameters	2667

A.6.6.17.2.3	Test Requirements	2672

A.6.6.17.3	Void	2672

A.6.6.17.3.1	Void	2672

A.6.6.17.3.2	Void	2672

A.6.6.17.3.3	Void	2672

A.6.6.18	SA event triggered reporting tests with concurrent gaps	2672

A.6.6.18.1	SA event triggered reporting tests for FR1 concurrent gaps with non-overalpping scenario for SSB-based measurements in both inter-frequency layers	2672

A.6.6.18.1.1	Test Purpose and Environment	2672

A.6.6.18.1.2	Test Requirements	2676

A.6.6.18.2	SA event triggered reporting tests for FR1 concurrent gap with partially partial overalpping scenario for SSB-based measurements in both inter-frequency layers	2676

A.6.6.18.2.1	Test Purpose and Environment	2676

A.6.6.18.2.2	Test Requirements	2680

A.6.6.18.3	SA NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1	2680

A.6.6.18.3.1	Test Purpose and Environment	2680

A.6.6.18.3.2	Test Requirements	2687

A.6.6.18.4	SA event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used	2687

A.6.6.18.4.1	Test Purpose and Environment	2687

A.6.6.18.4.2	Test Requirements	2691

A.6.6.19	SA event triggered reporting tests with NCSG	2691

A.6.6.19.1	SA event triggered reporting tests with NCSG under non-DRX in FR1	2691

A.6.6.19.1.1	Test purpose and Environment	2691

A.6.6.19.1.2	Test parameters	2692

A.6.6.19.1.3	Test Requirements	2696

A.6.6.19.2	SA event triggered reporting tests for FR1 with NCSG for inter-frequency measurement	2696

A.6.6.19.2.1	Test Purpose and Environment	2696

A.6.6.19.2.2	Test parameters	2696

A.6.6.19.2.3	Test Requirements	2700

A.6.6.19.3	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG	2700

A.6.6.19.3.1	Test Purpose and Environment	2700

A.6.6.19.3.2	Test parameters	2701

A.6.6.19.3.3	Test Requirements	2706

A.6.6.19.4	Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX	2706

A.6.6.19.4.1	Test purpose and Environment	2706

A.6.6.19.4.2	Test parameters	2706

A.6.6.19.4.3	Test Requirements	2709

A.6.6.20	UE Rx-Tx time difference measurement for propagation delay compensation	2709

A.6.6.20.1	Test purpose and environment	2709

A.6.6.20.2	Test requirements	2713

A.6.6.21	UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA	2713

A.6.6.21.1	Test purpose and environment	2713

A.6.6.21.2	Test requirements	2715

A.6.7	Measurement Performance requirements	2716

A.6.7.1	SS-RSRP	2716

A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2716

A.6.7.1.1.1	Test Purpose and Environment	2716

A.6.7.1.1.2	Test parameters	2716

A.6.7.1.1.3	Test Requirements	2721

A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2721

A.6.7.1.2.1	Test Purpose and Environment	2721

A.6.7.1.2.2	Test parameters	2721

A.6.7.1.2.3	Test Requirements	2725

A.6.7.1.3	Void	2725

A.6.7.2	SS-RSRQ	2725

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2725

A.6.7.2.1.1	Test Purpose and Environment	2725

A.6.7.2.1.2	Test Parameters	2726

A.6.7.2.1.3	Test Requirements	2731

A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2731

A.6.7.2.2.1	Test Purpose and Environment	2731

A.6.7.2.2.2	Test Parameters	2731

A.6.7.2.2.3	Test Requirements	2736

A.6.7.3	SS-SINR	2736

A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2736

A.6.7.3.1.1	Test Purpose and Environment	2736

A.6.7.3.1.2	Test Parameters	2737

A.6.7.3.1.3	Test Requirements	2741

A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2741

A.6.7.3.2.1	Test Purpose and Environment	2741

A.6.7.3.2.2	Test Parameters	2742

A.6.7.3.2.3	Test Requirements	2746

A.6.7.4	L1-RSRP measurement for beam reporting	2747

A.6.7.4.1	SSB based L1-RSRP measurement	2747

A.6.7.4.1.1	Test Purpose and Environment	2747

A.6.7.4.1.2	Test parameters	2747

A.6.7.4.1.3	Test Requirements	2751

A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2751

A.6.7.4.2.1	Test Purpose and Environment	2751

A.6.7.4.2.2	Test parameters	2752

A.6.7.4.2.3	Test Requirements	2756

A.6.7.5	E-UTRAN RSRP	2756

A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2756

A.6.7.5.1.1	Test Purpose and Environment	2756

A.6.7.5.1.2	Test parameters	2757

A.6.7.5.1.3	Test Requirements	2763

A.6.7.6	E-UTRAN RSRQ	2764

A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2764

A.6.7.6.1.1	Test Purpose and Environment	2764

A.6.7.6.1.2	Test parameters	2764

A.6.7.6.1.3	Test Requirements	2769

A.6.7.7	E-UTRAN RS-SINR	2770

A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2770

A.6.7.7.1.1	Test Purpose and Environment	2770

A.6.7.7.1.2	Test parameters	2770

A.6.7.7.1.3	Test Requirements	2776

A.6.7.8	CLI measurements	2777

A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell	2777

A.6.7.8.1.1	Test Purpose and Environment	2777

A.6.7.8.1.2	Test parameters	2777

A.6.7.8.1.3	Test Requirements	2783

A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell	2783

A.6.7.8.2.1	Test Purpose and Environment	2783

A.6.7.8.2.2	Test parameters	2784

A.6.7.8.2.3	Test Requirements	2787

A.6.7.9	L1-SINR measurement for beam reporting	2788

A.6.7.9.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2792

A.6.7.9.2.1	Test Purpose and Environment	2792

A.6.7.9.2.2	Test parameters	2793

A.6.7.9.2.3	Test Requirements	2798

A.6.7.9.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2798

A.6.7.9.3.1	Test Purpose and Environment	2798

A.6.7.9.3.2	Test parameters	2798

A.6.7.9.3.3	Test Requirements	2803

A.6.7.10	CSI-RSRP	2803

A.6.7.10.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2803

A.6.7.9.10.1	Test Purpose and Environment	2803

A.6.7.9.10.2	Test parameters	2804

A.6.7.10.1.3	Test Requirements	2809

A.6.7.10.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2809

A.6.7.9.10.1	Test Purpose and Environment	2809

A.6.7.10.2.2	Test parameters	2809

A.6.7.10.2.3	Test Requirements	2814

A.6.7.11	CSI-RSRQ	2814

A.6.7.11.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2814

A.6.7.11.1.1	Test Purpose and Environment	2814

A.6.7.11.1.2	Test Parameters	2814

A.6.7.11.1.3	Test Requirements	2819

A.6.7.11.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2819

A.6.7.11.2.1	Test Purpose and Environment	2819

A.6.7.11.2.2	Test Parameters	2819

A.6.7.11.2.3	Test Requirements	2825

A.6.7.12	CSI-SINR	2826

A.6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2826

A.6.7.12.1.1	Test Purpose and Environment	2826

A.6.7.12.1.2	Test Parameters	2826

A.6.7.12.1.3	Test Requirements	2830

A.6.7.12.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2831

A.6.7.12.2.1	Test Purpose and Environment	2831

A.6.7.12.2.2	Test Parameters	2831

A.6.7.12.2.3	Test Requirements	2837

A.6.7.13	RSTD measurements	2837

A.6.7.13.1	RSTD measurement accuracy test case for single positioning frequency layer	2837

A.6.7.13.1.1	Test purpose and Environment	2837

A.6.7.13.1.2	Test Requirements	2840

A.6.7.13.2	RSTD measurement accuracy test case for dual positioning frequency layer	2840

A.6.7.13.2.1	Test purpose and Environment	2840

A.6.7.13.2.2	Test Requirements	2844

A.6.7.13.3	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state	2844

A.6.7.13.3.1	Test purpose and Environment	2844

A.6.7.13.3.2	Test Requirements	2846

A.6.7.13.4	RSTD measurement accuracy test case with Rx TEG	2846

A.6.7.14	PRS-RSRP measurements	2850

A.6.7.14.1	SA: measurement accuracy with PRS in FR1	2850

A.6.7.14.1.1	Test Purpose and Environment	2850

A.6.7.14.1.2	Test parameters	2851

A.6.7.14.1.3	Test Requirements	2855

A.6.7.14.2	SA: measurement accuracy with PRS in FR1 with reduced sample number	2855

A.6.7.14.2.1	Test Purpose and Environment	2855

A.6.7.14.2.2	Test parameters	2855

A.6.7.14.2.3	Test Requirements	2858

A.6.7.14.3	Void	2859

A.6.7.14.3.1	Void	2859

A.6.7.14.3.2	Void	2859

A.6.7.14.3.3	Void	2859

A.6.7.15	UE Rx-Tx time difference measurements	2859

A.6.7.15.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA	2859

A.6.7.15.1.1	Test purpose and environment	2859

A.6.7.15.1.2	Test parameters	2859

A.6.7.15.1.3	Test requirements	2862

A.6.7.15.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR1 SA	2863

A.6.7.15.2.1	Test purpose and environment	2863

A.6.7.15.2.2	Test parameters	2863

A.6.7.15.2.3	Test requirements	2866

A.6.7.15.3	UE Rx-Tx time difference measurement accuracy with RxTx TEG	2866

A.6.7.15.3.1	Test purpose and environment	2866

A.6.7.15.3.2	Test parameters	2867

A.6.7.15.3.3	Test requirements	2870

A.6.7.16	PRS-RSRPP measurements	2870

A.6.7.16.1	SA: measurement accuracy with PRS in FR1	2870

A.6.7.16.1.1	Test Purpose and Environment	2870

A.6.7.16.1.2	Test parameters	2871

A.6.7.16.1.3	Test Requirements	2876

A.6.7.16.2	SA: measurement accuracy with reduced PRS samples in FR1	2876

A.6.7.16.2.1	Test Purpose and Environment	2876

A.6.7.16.2.2	Test parameters	2876

A.6.8	Measurement procedure in RRC_INACTIVE	2880

A.6.8.1	RSTD measurements	2880

A.6.8.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_INACTIVE state	2880

A.6.8.1.1.1	Test Purpose and Environment	2880

A.6.8.1.1.2	Test Requirements	2883

A.6.8.1.2	 NR RSTD measurement reporting delay test case with reduced number of samples in RRC_INACTIVE, FR1 SA	2883

A.6.8.1.2.1	Test Purpose and Environment	2883

A.6.8.1.2.2	Test Requirements	2888

A.6.8.2	PRS-RSRP measurements	2888

A.6.8.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	2888

A.6.8.2.1.1	Test purpose and Environment	2888

A.6.8.2.1.2	Test Requirements	2890

A.6.8.2.2	PRS-RSRP reporting delay test case with reduced number of samples in RRC_INACTIVE	2891

A.6.8.2.2.1	Test purpose and Environment	2891

A.6.8.2.2.2	Test Requirements	2894

A.6.8.3	UE Rx-Tx time difference measurements	2894

A.6.8.3.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	2894

A.6.8.3.1.1	Test purpose and environment	2894

A.6.8.3.1.2	Test requirements	2898

A.6.8.3.2	UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE, FR1 SA	2898

A.6.8.3.2.1	Test purpose and environment	2898

A.6.8.3.2.2	Test requirements	2901

A.6.8.4	PRS-RSRPP measurements	2901

A.6.8.4.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state	2901

A.6.8.4.1.1	Test purpose and Environment	2901

A.6.8.4.1.2	Test Requirements	2904

A.6.8.4.2	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state for reduced number of samples	2904

A.6.8.4.2.1	Test purpose and Environment	2904

A.6.8.4.2.2	Test Requirements	2906

A.6.9	Measurement performance requirements in RRC_INACTIVE	2906

A.6.9.1	RSTD measurements	2906

A.6.9.1.1	RSTD measurement accuracy test case for single positioning frequency layer in FR1 in RRC_INACTIVE state	2906

A.6.9.1.1.1	Test purpose and Environment	2906

A.6.9.1.1.2	Test Requirements	2908

A.6.9.1.2	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_INACTIVE state	2908

A.6.9.1.2.1	Test purpose and Environment	2908

A.6.9.1.2.2	Test Requirements	2910

A.6.9.2	PRS-RSRP measurements	2910

A.6.9.2.1	SA: measurement accuracy with PRS in FR1 in RRC_INACTIVE	2910

A.6.9.2.1.1	Test Purpose and Environment	2910

A.6.9.2.1.2	Test parameters	2911

A.6.9.2.1.3	Test Requirements	2916

A.6.9.2.2	SA: measurement accuracy with PRS in FR1 with reduced number of samples in RRC_INACTIVE state	2916

A.6.9.2.2.1	Test Purpose and Environment	2916

A.6.9.2.2.2	Test parameters	2916

A.6.9.2.2.3	Test Requirements	2919

A.6.9.3	UE Rx-Tx time difference measurements	2920

A.6.9.3.1.1	UE Rx-Tx time difference measurement accuracy in FR1 SA	2920

A.6.9.3.1.1.1	Test purpose and environment	2920

A.6.9.3.1.1.2 Test parameters	2920

A.6.9.3.1.1.3	Test requirements	2923

A.6.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples	2923

A.6.9.3.2.1	Test purpose and environment	2923

A.6.9.3.2.2	Test parameters	2923

A.6.9.3.2.3	Test requirements	2926

A.6.9.4	PRS-RSRPP measurements	2926

A.6.9.4.1	SA: PRS-RSRPP measurement accuracy in FR1 in RRC INACTIVE	2926

A.6.9.4.1.1	Test Purpose and Environment	2926

A.6.9.4.1.2	Test parameters	2926

A.6.9.4.1.3	Test Requirements	2931

A.6.9.4.2	SA: measurement accuracy with reduced PRS samples in FR1 in RRC INACTIVE	2931

A.6.9.4.2.1	Test Purpose and Environment	2931

A.6.9.4.2.2	Test parameters	2931

A.6.9.4.2.3	Test Requirements	2934

A.7	NR standalone tests with one or more NR cells in FR2	2934

A.7.1	SA: RRC_IDLE state mobility	2934

A.7.1.1	Cell re-selection to NR	2934

A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case	2934

A.7.1.1.1.1	Test Purpose and Environment	2934

A.7.1.1.1.2	Test Parameters	2934

A.7.1.1.1.3	Test Requirements	2937

A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case	2937

A.7.1.1.2.1	Test Purpose and Environment	2937

A.7.1.1.2.2	Test Parameters	2938

A.7.1.1.2.3	Test Requirements	2940

A.7.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2940

A.7.1.1.3.1	Test Purpose and Environment	2940

A.7.1.1.3.2	Test Parameters	2940

A.7.1.1.3.3	Test Requirements	2943

A.7.1.1.4	Cell reselection to FR2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2943

A.7.1.1.4.1	Test Purpose and Environment	2943

A.7.1.1.4.2	Test Parameters	2943

A.7.1.1.4.3	Test Requirements	2946

A.7.1.1.5	Cell reselection to FR2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2946

A.7.1.1.5.1	Test Purpose and Environment	2946

A.7.1.1.5.2	Test Parameters	2946

A.7.1.1.5.3	Test Requirements	2949

A.7.1.1.6	Cell reselection to FR2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2949

A.7.1.1.6.1	Test Purpose and Environment	2949

A.7.1.1.6.2	Test Parameters	2949

A.7.1.1.6.3	Test Requirements	2952

A.7.1.1.7	Cell reselection to FR2 intra-frequency NR case for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17	2952

A.7.1.1.7.1	Test Purpose and Environment	2952

A.7.1.1.7.2	Test Parameters	2952

A.7.1.1.7.3	Test Requirements	2956

A.7.2	SA: RRC_INACTIVE state mobility	2956

A.7.2.1	Small Data Transmission	2956

A.7.2.1.1	TA validation for CG-SDT in FR2	2956

A.7.2.1.1.1	Test Purpose and Environment	2956

A.7.2.1.1.2	Test Requirements	2960

A.7.3	RRC_CONNECTED state mobility	2960

A.7.3.1	Handover	2960

A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell	2960

A.7.3.1.1.1	Test Purpose and Environment	2960

A.7.3.1.1.2	Test Parameters	2960

A.7.3.1.1.3	Test Requirements	2963

A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell	2963

A.7.3.1.2.1	Test Purpose and Environment	2963

A.7.3.1.2.2	Test Parameters	2963

A.7.3.1.2.3	Test Requirements	2966

A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell	2966

A.7.3.1.3.1	Test Purpose and Environment	2966

A.7.3.1.3.2	Test Parameters	2966

A.7.3.1.3.3	Test Requirements	2967

A.7.3.1.4	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2	2968

A.7.3.1.4.1	Test Purpose and Environment	2968

A.7.3.1.4.2	Test Parameters	2968

A.7.3.1.4.3 Test Requirements	2975

A.7.3.1.5	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2	2975

A.7.3.1.5.1	Test Purpose and Environment	2975

A.7.3.1.5.2	Test Parameters	2975

A.7.3.1.5.3 Test Requirements	2982

A.7.3.1.6	Handover with PSCell from SA to EN-DC; unknown FR2 target cell	2982

A.7.3.1.6.1	Test Purpose and Environment	2982

A.7.3.1.6.2	Test Parameters	2982

A.7.3.1.6.3	Test Requirements	2987

A.7.3.1.7	HO with PSCell from FR1 NR-SA to EN-DC with known E-UTRA PCell and known FR2 PSCell	2987

A.7.3.1.7.1	Test purpose and environment	2987

A.7.3.1.7.2	Test Requirements	2993

A.7.3.1.8	NR PSCell change delay in HO with PSCell from NR-DC to NR-DC	2993

A.7.3.1.8.1	Test Purpose and Environment	2993

A.7.3.1.8.2	Test Requirements	2999

A.7.3.1.9	Intra-frequency handover from FR2-2 to FR2-2; unknown target cell	2999

A.7.3.1.9.1	Test Purpose and Environment	2999

A.7.3.1.9.2	Test Parameters	2999

A.7.3.1.9.3	Test Requirements	3002

A.7.3.1.10	Inter-frequency handover from FR2-2 to FR2-2; unknown target cell	3002

A.7.3.1.10.1	Test Purpose and Environment	3002

A.7.3.1.10.2	Test Parameters	3002

A.7.3.1.10.3	Test Requirements	3006

A.7.3.1.11	Inter-frequency handover from FR1 to FR2-2; unknown target cell	3006

A.7.3.1.11.1	Test Purpose and Environment	3006

A.7.3.1.11.2	Test Parameters	3006

A.7.3.1.11.3	Test Requirements	3010

A.7.3.2	RRC Connection Mobility Control	3010

A.7.3.2.1	SA: RRC Re-establishment	3010

A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	3010

A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	3013

A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	3016

A.7.3.2.1.3.1	Test Purpose and Environment	3016

A.7.3.2.1.3.2	Test Requirements	3018

A.7.3.2.1.4	Intra-frequency RRC Re-establishment in FR2-2	3019

A.7.3.2.1.4.1	Test Purpose and Environment	3019

A.7.3.2.1.4.2	Test Requirements	3021

A.7.3.2.1.5	Inter-frequency RRC Re-establishment in FR2-2	3022

A.7.3.2.1.5.1	Test Purpose and Environment	3022

A.7.3.2.1.5.2	Test Requirements	3025

A.7.3.2.1.6	Intra-frequency RRC Re-establishment in FR2-2 without serving cell timing	3025

A.7.3.2.1.6.1	Test Purpose and Environment	3025

A.7.3.2.1.6.2	Test Requirements	3027

A.7.3.2.2	Random Access	3028

A.7.3.2.2.1	4-step RA type c ontention based random access test in FR2 for NR Standalone	3028

A.7.3.2.2.2	4-step RA type n on-contention based random access test in FR2 for NR Standalone	3032

A.7.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	3036

A.7.3.2.2.4	2-step RA type n on-contention based random access test in FR2 for NR Standalone	3039

A.7.3.2.3	SA: RRC Connection Release with Redirection	3042

A.7.3.2.3.1	Redirection from NR in FR2 to NR in FR2	3042

A.7.3.3 Conditional Handover	3046

A.7.3.3.1	Intra-frequency conditional handover from FR2 to FR2	3046

A.7.3.3.1.1	Test Purpose and Environment	3046

A.7.3.3.1.2	Test Parameters	3046

A.7.3.3.1.2.3	Test Requirements	3049

A.7.3.3.2	Inter-frequency conditional handover from FR2 to FR2; unknown target cell	3049

A.7.3.3.2.1	Test Purpose and Environment	3049

A.7.3.3.2.2	Test Parameters	3049

A.7.3.3.2.3 Test Requirements	3051

A.7.4	Timing	3051

A.7.4.1	UE transmit timing	3051

A.7.4.1.1	NR UE Transmit Timing Test for FR2	3051

A.7.4.1.1.1	Test Purpose and environment	3051

A.7.4.1.1.2	Test requirements	3054

A.7.4.1.2	NR UE Transmit Timing Test for FR2-2	3055

A.7.4.1.2.1	Test Purpose and environment	3055

A.7.4.1.2.2	Test requirements	3059

A.7.4.2	UE timer accuracy	3060

A.7.4.3	Timing advance	3060

A.7.4.3.1	SA FR2 timing advance adjustment accuracy	3060

A.7.4.3.1.1	Test Purpose and Environment	3060

A.7.4.3.1.2	Test Parameters	3060

A.7.4.3.1.3 Test Requirements	3063

A.7.4.3.2	SA FR2-2 timing advance adjustment accuracy	3064

A.7.4.3.2.1	Test Purpose and Environment	3064

A.7.4.3.2.2	Test Parameters	3064

A.7.4.3.2.3	Test Requirements	3068

A.7.5	Signaling characteristics	3068

A.7.5.1	Radio link Monitoring	3068

A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3068

A.7.5.1.1.1	Test Purpose and Environment	3068

A.7.5.1.1.2	Test Requirements	3071

A.7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3072

A.7.5.1.2.1	Test Purpose and Environment	3072

A.7.5.1.2.2	Test Requirements	3076

A.7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	3077

A.7.5.1.3.1	Test Purpose and Environment	3077

A.7.5.1.3.2	Test Requirements	3081

A.7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	3081

A.7.5.1.4.1	Test Purpose and Environment	3081

A.7.5.1.4.2	Test Requirements	3086

A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	3086

A.7.5.1.5.1	Test Purpose and Environment	3086

A.7.5.1.5.2	Test Requirements	3091

A.7.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	3091

A.7.5.1.6.1	Test Purpose and Environment	3091

A.7.5.1.6.2	Test Requirements	3095

A.7.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	3095

A.7.5.1.7.1	Test Purpose and Environment	3095

A.7.5.1.7.2	Test Requirements	3099

A.7.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	3099

A.7.5.1.8.1	Test Purpose and Environment	3099

A.7.5.1.8.2	Test Requirements	3104

A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	3104

A.7.5.1.9.1	Test Purpose and Environment	3104

A.7.5.1.9.2	Test Requirements	3107

A.7.5.2	Interruption	3107

A.7.5.2.1	Interruptions during measurements on deactivated NR SCC in FR2	3107

A.7.5.2.1.1	Test Purpose and Environment	3107

A.7.5.2.1.2	Test Requirements	3110

A.7.5.2.2	SA interruptions at NR SRS carrier-based switching	3111

A.7.5.2.2.1	Test Purpose and Environment	3111

A.7.5.2.2.2	Test Parameters	3111

A.7.5.2.2.3	Test Requirements	3113

A.7.5.3	SCell Activation and Deactivation Delay	3113

A.7.5.3.1	SCell Activation and deactivation for SCell in FR2 intra-band in non-DRX	3113

A.7.5.3.1.1	Test Purpose and Environment	3113

A.7.5.3.1.2	Test Requirements	3115

A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2	3115

A.7.5.3.2.1	Test Purpose and Environment	3115

A.7.5.3.2.2	Test Requirements	3119

A.7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	3120

A.7.5.3.3.1	Test Purpose and Environment	3120

A.7.5.3.3.2	Test Requirements	3123

A.7.5.3.4	Direct SCell activation at SCell addition of known SCell in FR2	3124

A.7.5.3.4.1	Test Purpose and Environment	3124

A.7.5.3.4.2	Test Requirements	3127

A.7.5.3.5	Direct SCell activation at handover with known SCell in FR2	3128

A.7.5.3.5.1	Test Purpose and Environment	3128

A.7.5.3.5.2	Test Requirements	3131

A.7.5.3.6	PUCCH SCell activation and deactivation for FR1+FR2 inter-band with target SCell in FR2 and known	3132

A.7.5.3.6.1	Test Purpose and Environment	3132

A.7.5.3.6.2	Test Requirements	3136

A.7.5.3.7	PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR1 PCell	3137

A.7.5.3.7.1	Test Purpose and Environment	3137

A.7.5.3.7.2	Test Requirements	3141

A.7.5.3.8	SCell Activation and deactivation for known PUCCH SCell in FR2 inter-band in non-DRX	3142

A.7.5.3.8.1	Test Purpose and Environment	3142

A.7.5.3.8.2	Test Requirements	3146

A.7.5.3.9	PUCCH SCell Activation and deactivation of unknown SCell in FR2	3147

A.7.5.3.9.1	Test Purpose and Environment	3147

A.7.5.3.9.2	Test Requirements	3150

A.7.5.3.10	SCell Activation and deactivation of FR2 known PUCCH SCell and one FR2 unknown SCell with FR2 PCell	3151

A.7.5.3.10.1	Test Purpose and Environment	3151

A.7.5.3.10.2	Test Requirements	3155

A.7.5.3.11	PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR2 PCell	3156

A.7.5.3.11.1	PUCCH SCell activation with non-PUCCH SCell in a secondary PUCCH Group	3156

A.7.5.3.11.1.1	Test Purpose and Environment	3156

A.7.5.3.11.1.2	Test Requirements	3160

A.7.5.3.11.2	PUCCH SCell activation with non-PUCCH SCell in a primary PUCCH Group	3161

A.7.5.3.11.2.1	Test Purpose and Environment	3161

A.7.5.3.11.2.2	Test Requirements	3165

A.7.5.3.12	PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2	3166

A.7.5.3.12.1	Test Purpose and Environment	3166

A.7.5.3.12.2	Test Requirements	3170

A.7.5.3.13	SCell Activation for SCell in FR2 intra-band in non-DRX	3171

A.7.5.3.13.1	Test Purpose and Environment	3171

A.7.5.3.13.2	Test Requirements	3173

A.7.5.3.14	SCell Activation for known SCell in FR2 inter-band	3173

A.7.5.3.14.1	Test Purpose and Environment	3173

A.7.5.3.14.2	Test Requirements	3176

A.7.5.15	PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2	3177

A.7.5.15.3.1	Test Purpose and Environment	3177

A.7.5.3.2.2	Test Requirements	3179

A.7.5.4	Void	3180

A.7.5.5	Beam Failure Detection and Link recovery procedures	3180

A.7.5.5.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3180

A.7.5.5.1.1	Test Purpose and Environment	3180

A.7.5.5.1.2	Test Requirements	3184

A.7.5.5.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	3185

A.7.5.5.2.1	Test Purpose and Environment	3185

A.7.5.5.2.2	Test Requirements	3188

A.7.5.5.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	3189

A.7.5.5.3.1	Test Purpose and Environment	3189

A.7.5.5.3.2	Test Requirements	3193

A.7.5.5.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	3194

A.7.5.5.4.1	Test Purpose and Environment	3194

A.7.5.5.4.2	Test Requirements	3198

A.7.5.5.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3199

A.7.5.5.5.1	Test Purpose and Environment	3199

A.7.5.5.5.2	Test Requirements	3202

A.7.5.5.6	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	3203

A.7.5.5.6.1	Test Purpose and Environment	3203

A.7.5.5.6.2	Test Requirements	3207

A.7.5.5.7	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	3208

A.7.5.5.7.1	Test Purpose and Environment	3208

A.7.5.5.7.2	Test Requirements	3212

A.7.5.5.8	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode for UE fulfilling relaxed measurement criterion	3213

A.7.5.5.8.1	Test Purpose and Environment	3213

A.7.5.5.8.2	Test Requirements	3217

A.7.5.5.9	TRP specific Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	3217

A.7.5.5.9.1	Test Purpose and Environment	3217

A.7.5.5.9.2	Test Requirements	3223

A.7.5.5.10	TRP specific Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3223

A.7.5.5.10.1	Test Purpose and Environment	3223

A.7.5.5.10.2	Test Requirements	3229

A.7.5.5.11	Beam Failure Detection and Link Recovery Test for FR2-2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	3229

A.7.5.5.11.1	Test Purpose and Environment	3229

A.7.5.5.11.2	Test Requirements	3234

A.7.5.5.12	Beam Failure Detection and Link Recovery Test for FR2-2 PCell configured with CSI-RS-based BFD and LR in DRX mode	3234

A.7.5.5.12.1	Test Purpose and Environment	3234

A.7.5.5.12.2	Test Requirements	3239

A.7.5.5.13	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2-2 PCell configured with SSB-based BFD and LR in non-DRX mode	3239

A.7.5.5.13.1	Test Purpose and Environment	3239

A.7.5.5.13.2	Test Requirements	3243

A.7.5.6	Active BWP switch	3243

A.7.5.6.1	DCI-based and Timer-based Active BWP Switch	3243

A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of SCell with non-DRX in SA	3243

A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of SCell with non-DRX in SA	3248

A.7.5.6.1.3	NR FR2 DL active BWP switch with non-DRX in SA	3253

A.7.5.6.1.3.1	Test Purpose and Environment	3253

A.7.5.6.1.3.2	Test Requirements	3256

A.7.5.6.1.4	NR FR2-2- NR FR2-2 DL active BWP switch of SCell with non-DRX in SA	3256

A.7.5.6.1.4.1	Test Purpose and Environment	3256

A.7.5.6.1.4.2	Test Requirements	3261

A.7.5.6.2	RRC-based Active BWP Switch	3262

A.7.5.6.2.1.1	Test Purpose and Environment	3262

A.7.5.6.2.1.2	Test Requirements	3265

A.7.5.6.2.2	NR FR2-2 DL active BWP switch of PCell with non-DRX in SA	3266

A.7.5.6.2.2.1	Test Purpose and Environment	3266

A.7.5.6.2.2.2	Test Requirements	3270

A.7.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	3271

A.7.5.6.3.1.1	Test Purpose and Environment	3271

A.7.5.6.3.1.2	Test Requirements	3274

A.7.5.6.4	SCell dormancy switch	3274

A.7.5.6.4.1	NR FR2 PCell SCell dormancy switch of single FR2 SCell inside active time	3274

A.7.5.6.4.1.1	Test Purpose and Environment	3274

A.7.5.6.4.1.2	Test Requirements	3279

A.7.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time	3279

A.7.5.6.4.2.1	 Test Purpose and Environment	3279

A.7.5.6.4.2.2	 Test Requirements	3284

A.7.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	3284

A.7.5.6.5.1	Active BWP switch on multiple SCells with non-DRX in SA	3284

A.7.5.6.5.2	NR FR2-2 Active BWP switch on multiple SCells with non-DRX in SA	3287

A.7.5.6.5.2.1	Test Purpose and Environment	3287

A.7.5.6.5.2.2	Test Requirements	3291

A.7.5.7	PSCell addition and release delay	3292

A.7.5.7.1	Addition and Release Delay of known NR PSCell in FR2-2	3292

A.7.5.7.1.1	Test Purpose and Environment	3292

A.7.5.7.1.2	Test Requirements	3295

A.7.5.7.2	Addition and Release Delay of unknown NR PSCell in FR2-2	3295

A.7.5.7.2.1	Test Purpose and Environment	3295

A.7.5.7.2.2	Test Requirements	3298

A.7.5.8	Active TCI state switch delay	3298

A.7.5.8.1	MAC-CE based active TCI state switch	3298

A.7.5.8.2	RRC based active TCI state switch	3302

A.7.5.8.3	MAC-CE based active TCI state switch for HST FR2 scenario	3306

A.7.5.8.3.1	NR PCell FR2 HST active TCI state switch for a known TCI state	3306

A.7.5.8.3.1.1	Test Purpose and Environment	3306

A.7.5.8.3.1.2	Test Requirements	3310

A.7.5.9	Uplink spatial relation switch delay	3311

A.7.5.9.1.1.1	Test Purpose and Environment	3311

A.7.5.9.1.1.2	Test Requirements	3314

A.7.5.9.2	RRC based spatial relation switch	3314

A.7.5.9.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	3314

A.7.5.9.2.1.1	Test Purpose and Environment	3314

A.7.5.9.2.1.2	Test Requirements	3317

A.7.5.10	UE specific CBW change	3317

A.7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	3317

A.7.5.10.1.1	Test Purpose and Environment	3317

A.7.5.10.1.2	Test Requirements	3320

A.7.5.11	UE UL carrier RRC reconfiguration Delay	3321

A.7.5.11.1	UE UL carrier RRC reconfiguration Delay	3321

A.7.5.11.1.1	Test Purpose and Environment	3321

A.7.5.11.1.2	Test Requirements	3324

A.7.5.12	Conditional PSCell addition and release delay (FR2 SA)	3324

A.7.5.12.1	Addition and Release Delay of PSCell	3324

A.7.5.12.1.1	Test purpose and environment	3324

A.7.5.12.1.2	Test Parameters	3324

A.7.5.12.1.3	Test Requirements	3327

A.7.5.13	 Unified TCI state switching delay	3327

A.7.5.13.1	MAC-CE based active joint TCI state switching	3327

A.7.5.13.1.1	NR PCell FR2 active joint TCI state switch for a known TCI state	3327

A.7.5.13.1.1.1	Test Purpose and Environment	3327

A.7.5.13.1.1.2	Test parameters	3328

A.7.5.13.1.1.3	Test Requirements	3330

A.7.5.13.2	 MAC-CE based active uplink TCI state switch	3330

A.7.5.13.2.1	 NR FR2 PCell uplink TCI state switch for a known TCI state	3330

A.7.5.13.2.1.1	Test Purpose and Environment	3330

A.7.5.13.2.1.2	Test parameters	3331

A.7.5.13.2.1.3	Test Requirements	3333

A.7.5.13.3	MAC-CE based active downlink TCI state switch	3333

A.7.5.13.3.1	NR PCell FR2 active downlink TCI state switch to cell with additional PCI for a known TCI state	3333

A.7.5.13.3.1.1	Test Purpose and Environment	3333

A.7.5.13.3.1.2	Test Parameters	3334

A.7.5.13.3.1.3	Test Requirements	3337

A.7.6	Measurement procedure	3337

A.7.6.1	Intra-frequency Measurements	3337

A.7.6.1.1	SA event triggered reporting test without gap under non-DRX	3337

A.7.6.1.1.1	Test purpose and Environment	3337

A.7.6.1.1.2	Test Requirements	3341

A.7.6.1.2	SA event triggered reporting test without gap under DRX	3341

A.7.6.1.2.1	Test purpose and Environment	3341

A.7.6.1.2.2	Test Requirements	3344

A.7.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	3345

A.7.6.1.3.1	Test purpose and Environment	3345

A.7.6.1.3.2	Test Requirements	3348

A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	3348

A.7.6.1.4.1	Test purpose and Environment	3348

A.7.6.1.4.2	Test Requirements	3351

A.7.6.1.5	SA event triggered reporting test without gap under non-DRX for UE configured with highSpeedMeasCA-Scell-r17	3352

A.7.6.1.5.1	Test purpose and Environment	3352

A.7.6.1.5.2	Test Requirements	3355

A.7.6.1.6	SA event triggered reporting test without gap under non-DRX for FR2-2	3355

A.7.6.1.6.1	Test purpose and Environment	3355

A.7.6.1.6.2	Test Requirements	3358

A.7.6.1.7	SA event triggered reporting test without gap under DRX for FR2-2	3359

A.7.6.1.7.1	Test purpose and Environment	3359

A.7.6.1.7.2	Test Requirements	3361

A.7.6.1.8	SA event triggered reporting test with per-UE gaps under non-DRX for FR2-2	3362

A.7.6.1.8.1	Test purpose and Environment	3362

A.7.6.1.8.2	Test Requirements	3365

A.7.6.1.9	SA event triggered reporting test with per-UE gaps under DRX for FR2-2	3366

A.7.6.1.9.1	Test purpose and Environment	3366

A.7.6.1.9.2	Test Requirements	3368

A.7.6.1.10	SA event triggered reporting test with SSB time index detection without gap under non-DRX for FR2-2	3369

A.7.6.1.10.1	Test purpose and Environment	3369

A.7.6.1.10.2	Test Requirements	3372

A.7.6.1.11	SA event triggered reporting test with SSB time index detection with per-UE gaps under non-DRX for FR2-2	3373

A.7.6.1.11.1	Test purpose and Environment	3373

A.7.6.1.11.2	Test Requirements	3375

A.7.6.2	Inter-frequency Measurements	3376

A.7.6.2.1	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR2)	3376

A.7.6.2.1.1	Test Purpose and Environment	3376

A.7.6.2.1.2	Test Requirements	3379

A.7.6.2.2	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (Pcell in FR2)	3379

A.7.6.2.2.1	Test Purpose and Environment	3379

A.7.6.2.2.2	Test Requirements	3383

A.7.6.2.3	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR2)	3383

A.7.6.2.3.1	Test Purpose and Environment	3383

A.7.6.2.3.2	Test Requirements	3387

A.7.6.2.4	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (Pcell in FR2)	3387

A.7.6.2.4.1	Test Purpose and Environment	3387

A.7.6.2.4.2	Test Requirements	3391

A.7.6.2.5	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)	3391

A.7.6.2.5.1	Test Purpose and Environment	3391

A.7.6.2.5.2	Test Requirements	3395

A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (Pcell in FR1)	3396

A.7.6.2.6.1	Test Purpose and Environment	3396

A.7.6.2.6.2	Test Requirements	3400

A.7.6.2.7	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)	3401

A.7.6.2.7.1	Test Purpose and Environment	3401

A.7.6.2.7.2	Test Requirements	3405

A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)	3406

A.7.6.2.8.1	Test Purpose and Environment	3406

A.7.6.2.8.2	Test Requirements	3410

A.7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	3411

A.7.6.2.9.1	Test Purpose and Environment	3411

A.7.6.2.9.2	Test Requirements	3415

A.7.6.2.10	SA event triggered reporting test without gap under non-DRX	3415

A.7.6.2.10.1	Test Purpose and Environment	3415

A.7.6.2.10.2	Test Requirements	3417

A.7.6.2.11	SA event triggered reporting test without gap under DRX	3417

A.7.6.2.11.1	Test Purpose and Environment	3417

A.7.6.2.11.2	Test Requirements	3420

A.7.6.2.12	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is not used (PCell in FR2-2)	3420

A.7.6.2.12.1	Test Purpose and Environment	3420

A.7.6.2.12.2	Test Requirements	3424

A.7.6.2.13	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is used (PCell in FR2-2)	3425

A.7.6.2.13.1	Test Purpose and Environment	3425

A.7.6.2.13.2	Test Requirements	3429

A.7.6.2.14	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is not used (PCell in FR2-2)	3430

A.7.6.2.14.1	Test Purpose and Environment	3430

A.7.6.2.14.2	Test Requirements	3434

A.7.6.2.15	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is used (PCell in FR2-2)	3435

A.7.6.2.15.1	Test Purpose and Environment	3435

A.7.6.2.15.2	Test Requirements	3439

A.7.6.2.16	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is not used (PCell in FR1)	3440

A.7.6.2.16.1	Test Purpose and Environment	3440

A.7.6.2.16.2	Test Requirements	3446

A.7.6.2.17	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is used (PCell in FR1)	3446

A.7.6.2.17.1	Test Purpose and Environment	3446

A.7.6.2.17.2	Test Requirements	3453

A.7.6.2.18	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is not used (PCell in FR1)	3454

A.7.6.2.18.1	Test Purpose and Environment	3454

A.7.6.2.18.2	Test Requirements	3459

A.7.6.2.19	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is used (PCell in FR1)	3460

A.7.6.2.19.1	Test Purpose and Environment	3460

A.7.6.2.19.2	Test Requirements	3468

A.7.6.3	L1-RSRP measurement for beam reporting	3469

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	3469

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	3469

A.7.6.3.1.1	Test Purpose and Environment	3469

A.7.6.3.1.2	Test parameters	3469

A.7.6.3.1.3	Test Requirements	3471

A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used	3471

A.7.6.3.2.1	Test Purpose and Environment	3471

A.7.6.3.2.2	Test parameters	3472

A.7.6.3.2.3	Test Requirements	3474

A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	3474

A.7.6.3.3.1	Test Purpose and Environment	3474

A.7.6.3.3.2	Test parameters	3475

A.7.6.3.3.3	Test Requirements	3477

A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	3478

A.7.6.3.4.1	Test Purpose and Environment	3478

A.7.6.3.4.2	Test parameters	3478

A.7.6.3.3.3	Test Requirements	3480

A.7.6.3.5	SSB based L1-RSRP measurement when DRX is used for power class 6 UE configured with highSpeedMeasFlagFR2-r17	3481

A.7.6.3.5.1	Test Purpose and Environment	3481

A.7.6.3.5.2	Test parameters	3481

A.7.6.3.5.3	Test Requirements	3483

A.7.6.3.6	Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used	3483

A.7.6.3.6.1	Test Purpose and Environment	3483

A.7.6.3.6.2	Test parameters	3484

A.7.6.3.6.3	Test Requirements	3487

A.7.6.3.7	SSB based L1-RSRP measurement for FR2-2 when DRX is used	3487

A.7.6.3.7.1	Test Purpose and Environment	3487

A.7.6.3.7.2	Test parameters	3488

A.7.6.3.7.3	Test Requirements	3491

A.7.6.4	CLI measurements	3492

A.7.6.4.1	SRS-RSRP measurement with non-DRX	3492

A.7.6.4.1.1	Test Purpose and Environment	3492

A.7.6.4.1.2	Test Parameters	3492

A.7.6.4.1.3	Test Requirements	3494

A.7.6.4.2	CLI-RSSI measurement with non-DRX	3494

A.7.6.4.2.1	Test Purpose and Environment	3494

A.7.6.4.2.2	Test Parameters	3495

A.7.6.4.2.3	Test Requirements	3496

A.7.6.5.1   SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2)	3497

A.7.6.5.1.1	Test Purpose and Environment	3497

A.7.6.5.1.2	Test Requirements	3500

A.7.6.6	L1-SINR measurement for beam reporting	3500

A.7.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	3502

A.7.6.6.2.1	Test Purpose and Environment	3502

A.7.6.6.2.2	Test parameters	3503

A.7.6.6.2.3	Test Requirements	3505

A.7.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	3505

A.7.6.6.3.1	Test Purpose and Environment	3505

A.7.6.6.3.2	Test parameters	3506

A.7.6.6.3.3	Test Requirements	3508

A.7.6.7	CSI-RS based intra-frequency Measurements	3508

A.7.6.7.1	SA event triggered reporting test without gap under DRX for CSI-RS based intra-frequency measurement	3508

A.7.6.7.1.1	Test purpose and Environment	3508

A.7.6.7.1.2	Test Requirements	3511

A.7.6.8	CSI-RS based inter-frequency Measurements	3511

A.7.6.8.1	SA event triggered reporting tests for FR2 CSI-RS based measurement when non-DRX is used (PCell in FR2)	3511

A.7.6.8.1.1	Test Purpose and Environment	3511

A.7.6.8.1.2	Test Requirements	3514

A.7.6.9	RSTD measurements	3514

A.7.6.9.1	 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA	3514

A.7.6.9.1.1	Test Purpose and Environment	3514

A.7.6.9.1.2	Test Requirements	3522

A.7.6.9.2	 NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR2 SA	3522

A.7.6.9.2.1	Test Purpose and Environment	3522

A.7.6.9.2.2	Test Requirements	3530

A.7.6.9.3	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA	3530

A.7.6.9.3.1	Test Purpose and Environment	3530

A.7.6.9.3.2	Test Requirements	3535

A.7.6.9.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA without measurement gap	3535

A.7.6.9.4.1	Test Purpose and Environment	3535

A.7.6.9.4.2	Test Requirements	3539

A.7.6.9.5	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_CONNECTED state with Rx TEG	3539

A.7.6.9.5.1	Test Purpose and Environment	3539

A.7.6.9.5.2	Test Requirements	3543

A.7.6.10 PRS-RSRP measurements	3543

A.7.6.10.1 PRS-RSRP reporting delay test case for single positioning frequency layer	3543

A.7.6.10.1.1	Test Purpose and Environment	3543

A.7.6.10.1.2	Test Requirements	3547

A.7.6.10.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	3547

A.7.6.10.2.1	Test Purpose and Environment	3547

A.7.6.10.2.2	Test Requirements	3551

A.7.6.10.3	PRS-RSRP reporting delay test case for reduced number of samples	3551

A.7.6.10.3.1	Test Purpose and Environment	3551

A.7.6.10.3.2	Test Requirements	3555

A.7.6.10.4	PRS-RSRP reporting delay test case for single positioning frequency layer outside MG	3555

A.7.6.10.4.1	Test Purpose and Environment	3555

A.7.6.10.4.2	Test Requirements	3559

A.7.6.11	UE Rx-Tx time difference measurements	3559

A.7.6.11.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	3559

A.7.6.11.1.1	Test purpose and environment	3559

A.7.6.11.1.2	Test requirements	3563

A.7.6.11.2	UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA	3563

A.7.6.11.2.1	Test purpose and environment	3563

A.7.6.11.2.2	Test requirements	3567

A.7.6.11.3	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA with reduced sample number	3567

A.7.6.11.3.1	Test purpose and environment	3567

A.7.6.11.3.2	Test requirements	3569

A.7.6.11.4	UE Rx-Tx time difference measurements without gaps in FR2 SA	3570

A.7.6.11.4.1	Test purpose and environment	3570

A.7.6.11.4.2	Test requirements	3572

A.7.6.11.5	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA with multiple RxTx TEGs	3573

A.7.6.11.5.1	Test purpose and environment	3573

A.7.6.11.5.2	Test requirements	3575

A.7.6.12	PRS-RSRPP measurements	3576

A.7.6.12.1 PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_CONNECTED state	3576

A.7.6.12.1.1	Test Purpose and Environment	3576

A.7.6.12.1.2	Test Requirements	3578

A.7.6.12.2	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_CONNECTED state	3579

A.7.6.12.2.1	Test Purpose and Environment	3579

A.7.6.12.2.2	Test Requirements	3581

A.7.6.12.3	PRS-RSRPP reporting delay test case for gapless measurement in FR2	3581

A.7.6.12.3.1	Test Purpose and Environment	3581

A.7.6.12.3.2	Test Requirements	3585

A.7.6.13	UE Rx-Tx time difference measurements for PDC	3585

A.7.6.13.1	UE Rx-Tx time difference measurement for propagation delay compensation using PRS in FR2	3585

A.7.6.13.1.1	Test purpose and environment13	3585

A.7.6.13.1.2	Test requirements	3587

A.7.6.13.2	UE Rx-Tx time difference measurement for propagation delay compensation using TRS in FR2	3588

A.7.6.13.2.1	Test purpose and environment	3588

A.7.6.13.2.2	Test requirements	3589

A.7.6.14	SA event triggered reporting tests with Pre-MG	3590

A.7.6.14.1	Intra-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MG in FR2	3590

A.7.6.14.1.1	Test purpose and Environment	3590

A.7.6.14.1.2	Test parameters	3590

A.7.6.14.1.3	Test Requirements	3592

A.7.6.14.2	Intra-frequency measurement test with SA event triggered reporting tests: with network-controlled activation/deactivation of Pre-MG in FR2	3593

A.7.6.14.2.1	Test purpose and Environment	3593

A.7.6.14.2.2	Test parameters	3593

A.7.6.14.2.3	Test Requirements	3595

A.7.6.15	SA event triggered reporting tests with concurrent gaps	3596

A.7.6.15.1	SA event triggered reporting tests For FR2 with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements	3596

A.7.6.15.1.1	Test Purpose and Environment	3596

A.7.6.15.1.2	Test Requirements	3599

A.7.6.15.2	SA event triggered reporting tests For FR2 with concurrent measurement gaps without SSB time index detection when DRX is not used (PCell in FR2)	3599

A.7.6.15.2.1	Test Purpose and Environment	3599

A.7.6.15.2.2	Test Requirements	3602

A.7.6.15.3	SA event triggered reporting tests for FR2 concurrent gap with partially partial overalpping scenario for SSB-based measurements and PRS-based measurement	3602

A.7.6.15.3.1	Test Purpose and Environment	3602

A.7.6.15.3.2	Test Requirements	3606

A.7.6.16	SA event triggered reporting tests with NCSG	3606

A.7.6.16.1	SA event triggered reporting test with per-UE NCSG under non-DRX	3606

A.7.6.16.1.1	Test purpose and Environment	3606

A.7.6.16.1.2	Test Requirements	3610

A.7.6.16.2	SA event triggered reporting tests on inter-frequency measurement with NCSG for FR2 when DRX is not used (PCell in FR2)	3610

A.7.6.16.2.1	Test Purpose and Environment	3610

A.7.6.16.2.2	Test Requirements	3613

A.7.6.16.3	Event triggered reporting test on deactivated Scell measurement via NCSG in FR2 in non-DRX	3613

A.7.6.16.3.1	Test Purpose and Environment	3613

A.7.6.16.3.2	Test Requirements	3616

A.7.7	Measurement Performance requirements	3616

A.7.7.1	SS-RSRP	3617

A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3617

A.7.7.1.1.1	Test Purpose and Environment	3617

A.7.7.1.1.2	Test parameters	3617

A.7.7.1.1.3	Test Requirements	3621

A.7.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3621

A.7.7.1.2.1	Test Purpose and Environment	3621

A.7.7.1.2.2	Test parameters	3622

A.7.7.1.2.3	Test Requirements	3626

A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	3627

A.7.7.1.3.1	Test Purpose and Environment	3627

A.7.7.1.3.2	Test parameters	3627

A.7.7.1.3.3	Test Requirements	3631

A.7.7.2	SS-RSRQ	3631

A.7.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	3631

A.7.7.2.1.1	Test Purpose and Environment	3631

A.7.7.2.1.2	Test Parameters	3631

A.7.7.2.1.3	Test Requirements	3633

A.7.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3633

A.7.7.2.2.1	Test Purpose and Environment	3633

A.7.7.2.2.2	Test Parameters	3633

A.7.7.2.2.3	Test Requirements	3635

A.7.7.3	SS-SINR	3635

A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3635

A.7.7.3.1.1	Test Purpose and Environment	3635

A.7.7.3.1.2	Test Parameters	3635

A.7.7.3.1.3	Test Requirements	3637

A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3637

A.7.7.3.2.1	Test Purpose and Environment	3637

A.7.7.3.2.2	Test Parameters	3637

A.7.7.3.2.3	Test Requirements	3639

A.7.7.4	L1-RSRP measurement for beam reporting	3639

A.7.7.4.1	SSB based L1-RSRP measurement	3639

A.7.7.4.1.1	Test Purpose and Environment	3639

A.7.7.4.1.2	Test parameters	3640

A.7.7.4.1.3	Test Requirements	3642

A.7.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	3643

A.7.7.4.2.1	Test Purpose and Environment	3643

A.7.7.4.2.2	Test parameters	3643

A.7.7.4.2.3	Test Requirements	3645

A.7.7.5	CLI measurements	3646

A.7.7.5.1	SA SRS-RSRP measurement accuracy with FR2 serving cell	3646

A.7.7.5.1.1	Test Purpose and Environment	3646

A.7.7.5.1.2	Test parameters	3646

A.7.7.5.1.3	Test Requirements	3649

A.7.7.5.2	SA CLI-RSSI measurement accuracy with FR2 serving cell	3650

A.7.7.5.2.1	Test Purpose and Environment	3650

A.7.7.5.2.2	Test parameters	3650

A.7.7.5.2.3	Test Requirements	3652

A.7.7.6	L1-SINR measurement for beam reporting	3653

A.7.7.6.1.1	Test Purpose and Environment	3653

A.7.7.6.1.2	Test parameters	3653

A.7.7.6.1.3	Test Requirements	3655

A.7.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	3656

A.7.7.6.2.1	Test Purpose and Environment	3656

A.7.7.6.2.2	Test parameters	3656

A.7.7.6.2.3	Test Requirements	3658

A.7.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	3659

A.7.7.6.3.1	Test Purpose and Environment	3659

A.7.7.6.3.2	Test parameters	3659

A.7.7.6.3.3	Test Requirements	3661

A.7.7.7	CSI-RSRP	3662

A.7.7.7.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3662

A.7.7.7.1.1	Test Purpose and Environment	3662

A.7.7.7.1.2	Test parameters	3662

A.7.7.7.1.3	Test Requirements	3665

A.7.7.7.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3666

A.7.7.7.2.1	Test Purpose and Environment	3666

A.7.7.7.2.2	Test parameters	3666

A.7.7.7.2.3	Test Requirements	3669

A.7.7.8	CSI-RSRQ	3669

A.7.7.8.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	3669

A.7.7.8.1.1	Test Purpose and Environment	3669

A.7.7.8.1.2	Test Parameters	3670

A.7.7.8.1.3	Test Requirements	3672

A.7.7.8.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3672

A.7.7.8.2.1	Test Purpose and Environment	3672

A.7.7.8.2.2	Test Parameters	3672

A.7.7.8.2.3	Test Requirements	3674

A.7.7.9	CSI-SINR	3674

A.7.7.9.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3674

A.7.7.9.1.1	Test Purpose and Environment	3674

A.7.7.9.1.2	Test Parameters	3674

A.7.7.9.1.3	Test Requirements	3676

A.7.7.9.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3676

A.7.7.9.2.1	Test Purpose and Environment	3676

A.7.7.9.2.2	Test Parameters	3677

A.7.7.9.2.3	Test Requirements	3679

A.7.7.10	RSTD measurements	3679

A.7.7.10.1	RSTD measurement accuracy test case for single positioning frequency layer	3679

A.7.7.10.1.1	Test purpose and Environment	3679

A.7.7.10.1.2	Test Requirements	3682

A.7.7.10.2	RSTD measurement accuracy test case for dual positioning frequency layer	3682

A.7.7.10.2.1	Test purpose and Environment	3682

A.7.7.10.2.2	Test Requirements	3686

A.7.7.10.3	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR2 in RRC_CONNECTED state	3687

A.7.7.10.3.1	Test purpose and Environment	3687

A.7.7.10.3.2	Test Requirements	3688

A.7.7.10.4	RSTD measurement accuracy test case with Rx TEG	3689

A.7.7.10.4.1	Test purpose and Environment	3689

A.7.7.10.4.2	Test Requirements	3691

A.7.7.11	PRS-RSRP measurements	3691

A.7.7.11.1	SA measurement accuracy with PRS in FR2	3691

A.7.7.11.1.1	Test Purpose and Environment	3691

A.7.7.11.1.2	Test parameters	3691

A.7.7.11.1.3	Test Requirements	3695

A.7.7.11.2	SA measurement accuracy with PRS in FR2 with reduced sample number	3696

A.7.7.11.2.1	Test Purpose and Environment	3696

A.7.7.11.2.2	Test parameters	3696

A.7.7.11.2.3	Test Requirements	3698

A.7.7.12	UE Rx-Tx time difference measurements	3699

A.7.7.12.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA	3699

A.7.7.12.1.1	Test purpose and environment	3699

A.7.7.12.1.2	Test parameters	3699

A.7.7.12.1.3	Test requirements	3702

A.7.7.12.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA	3702

A.7.7.12.2.1	Test purpose and environment	3702

A.7.7.12.2.2	Test parameters	3703

A.7.7.12.2.3	Test requirements	3706

A.7.7.12.3	UE Rx-Tx time difference measurement accuracy with RxTx TEG	3706

A.7.7.12.3.1	Test purpose and environment	3706

A.7.7.12.3.2	Test parameters	3706

A.7.7.12.3.3	Test requirements	3709

A.7.7.13	PRS-RSRPP measurements	3709

A.7.7.13.1	SA measurement accuracy with PRS in FR2	3709

A.7.7.13.1.1	Test Purpose and Environment	3709

A.7.7.13.1.2	Test parameters	3709

A.7.7.13.1.3	Test Requirements	3712

A.7.7.13.2	SA measurement accuracy with reduced PRS samples in FR2	3713

A.7.7.13.2.1	Test Purpose and Environment	3713

A.7.7.13.2.2	Test parameters	3713

A.7.7.13.2.3	Test Requirements	3715

A.7.8	Measurement procedure in RRC_INACTIVE	3716

A.7.8.1	RSTD measurements	3716

A.7.8.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_INACTIVE state	3716

A.7.8.1.1.1	Test Purpose and Environment	3716

A.7.8.1.1.2	Test Requirements	3719

A.7.8.1.2	NR RSTD measurement reporting delay test case with reduced number of samples in RRC_INACTIVE, FR1 SA	3719

A.7.8.1.2.1	Test Purpose and Environment	3719

A.7.8.1.2.2	Test Requirements	3725

A.7.8.2	PRS-RSRP measurements	3726

A.7.8.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	3726

A.7.8.2.1.1	Test Purpose and Environment	3726

A.7.8.2.1.2	Test Requirements	3730

A.7.8.2.2	PRS-RSRP reporting delay test case with reduced number of samples in RRC_INACTIVE	3730

A.7.8.2.2.1	Test purpose and Environment	3730

A.7.8.2.2.2	Test Requirements	3734

A.7.8.3	UE Rx-Tx time difference measurements	3734

A.7.8.3.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	3734

A.7.8.3.1.1	Test purpose and environment	3734

A.7.8.3.1.2	Test requirements	3738

A.7.8.3.2	UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE, FR2 SA	3738

A.7.8.3.2.1	Test purpose and environment	3738

A.7.8.3.2.2	Test requirements	3740

A.7.8.4	PRS-RSRPP measurements	3741

A.7.8.4.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_INACTIVE state	3741

A.7.8.4.1.1	Test Purpose and Environment	3741

A.7.8.4.1.2	Test Requirements	3743

A.7.8.4.2	PRS-RSRPP reporting delay test with reduced number of samples for single positioning frequency layer in FR2 in RRC_INACTIVE state	3743

A.7.8.4.2.1	Test Purpose and Environment	3743

A.7.8.4.2.2	Test Requirements	3746

A.7.9	Measurement performance requirements in RRC_INACTIVE	3746

A.7.9.1	RSTD measurements	3746

A.7.9.1.1	RSTD measurement accuracy test case for single positioning frequency layer in FR2 in RRC_INACTIVE state	3746

A.7.9.1.1.1	Test purpose and Environment	3746

A.7.9.1.1.2	Test Requirements	3748

A.7.9.1.2	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR2 in RRC_INACTIVE state	3748

A.7.9.1.2.1	Test purpose and Environment	3748

A.7.9.1.2.2	Test Requirements	3750

A.7.9.2	PRS-RSRP measurements	3750

A.7.9.2.1	SA measurement accuracy with PRS in FR2 in RRC_INACTIVE	3750

A.7.9.2.1.1	Test Purpose and Environment	3750

A.7.9.2.1.2	Test parameters	3750

A.7.9.2.1.3	Test Requirements	3752

A.7.9.2.2	PRS-RSRP measurements with reduced number of sample in RRC_INACTIVE	3753

A.7.9.2.2.1	Test Purpose and Environment	3753

A.7.9.2.2.2	Test parameters	3753

A.7.9.2.2.3	Test Requirements	3755

A.7.9.3	UE Rx-Tx time difference measurements	3756

A.7.9.3.1	UE Rx-Tx time difference measurements in RRC_INACTIVE	3756

A.7.9.3.1.1	Test purpose and environment	3756

A.7.9.3.1.2	Test parameters	3756

A.7.9.3.1.3	Test requirements	3758

A.7.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA	3758

A.7.9.3.2.1	Test purpose and environment	3758

A.7.9.3.2.2	Test parameters	3758

A.7.9.3.2.3	Test requirements	3761

A.7.9.4	PRS-RSRPP measurements	3761

A.7.9.4.1	SA measurement accuracy in FR2 in RRC INACTIVE	3761

A.7.9.4.1.1	Test Purpose and Environment	3761

A.7.9.4.1.2	Test parameters	3761

A.7.9.4.1.3	Test Requirements	3763

A.7.9.4.2	SA measurement accuracy with reduced PRS samples in FR2 in RRC INACTIVE	3764

A.7.9.4.2.1	Test Purpose and Environment	3764

A.7.9.4.2.2	Test parameters	3764

A.7.9.4.2.3	Test Requirements	3766

A.8	E-UTRA standalone tests for NR RRM	3758

A.8.1	Void	3758

A.8.2	RRC_IDLE state mobility	3758

A.8.2.1	Inter-RAT NR Cell re-selection	3758

A.8.2.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	3758

A.8.2.1.1.1	Test Purpose and Environment	3758

A.8.2.1.1.2	Test Requirements	3763

A.8.2.1.2	E-UTRA Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	3764

A.8.2.1.2.1	Test Purpose and Environment	3764

A.8.2.1.2.2	Test Requirements	3768

A.8.2.2	E-UTRA – NR Inter-RAT Early Measruement Reporting	3769

A.8.2.2.1	E-UTRA – NR Early Measurement Reporting for NR in FR1	3769

A.8.2.2.1.1	Test Purpose and Environment	3769

A.8.2.2.1.2	Test Requirements	3773

A.8.2.2.2	E-UTRA – NR Early Measurement Reporting for NR in FR2	3773

A.8.2.2.2.1	Test Purpose and Environment	3773

A.8.2.2.2.2	Test Requirements	3776

A.8.3	RRC_CONNECTED state mobility	3777

A.8.3.1	Handover	3777

A.8.3.1.1	E-UTRAN - NR handover in FR1	3777

A.8.3.1.1.1	Test Purpose and Environment	3777

A.8.3.1.1.2	Test Requirements	3782

A.8.4	Measurement procedure	3782

A.8.4.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay	3782

A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX	3782

A.8.4.1.1.1	Test Purpose and Environment	3782

A.8.4.1.1.2	Test Requirements	3784

A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX	3785

A.8.4.1.2.1	Test Purpose and Environment	3785

A.8.4.1.2.2	Test Requirements	3786

A.8.4.2	E-UTRA – NR Inter-RAT Measurements	3786

A.8.4.2.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	3786

A.8.4.2.1.1	Test Purpose and Environment	3786

A.8.4.2.1.2	Test Requirements	3791

A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	3791

A.8.4.2.2.1	Test Purpose and Environment	3791

A.8.4.2.2.2	Test Requirements	3795

A.8.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	3795

A.8.4.2.3.1	Test Purpose and Environment	3795

A.8.4.2.3.2	Test Requirements	3800

A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	3800

A.8.4.2.4.1	Test Purpose and Environment	3800

A.8.4.2.4.2	Test Requirements	3804

A.8.4.2.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	3804

A.8.4.2.5.1	Test Purpose and Environment	3804

A.8.4.2.5.2	Test Requirements	3806

A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	3807

A.8.4.2.6.1	Test Purpose and Environment	3807

A.8.4.2.6.2	Test Requirements	3809

A.8.4.2.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	3810

A.8.4.2.7.1	Test Purpose and Environment	3810

A.8.4.2.7.2	Test Requirements	3812

A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	3813

A.8.4.2.8.1	Test Purpose and Environment	3813

A.8.4.2.8.2	Test Requirements	3815

A.8.4.2.9	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	3816

A.8.4.2.9.1	Test Purpose and Environment	3816

A.8.4.2.9.2	Test Requirements	3820

A.8.5	Measurement performance	3820

A.8.5.1	SFTD accuracy	3820

A.8.5.1.1	SFTD accuracy	3820

A.8.5.1.1.1	Test Purpose	3820

A.8.5.1.1.2	Test Environment	3821

A.8.5.1.1.3	Test Requirements	3827

A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements	3827

A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	3827

A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	3832

A.8.5.2.1.2.1	Test Purpose and Environment	3832

A.8.5.2.1.2.2	Test Parameters	3832

A.8.5.2.1.2.3	Test Requirements	3834

A.8.5.2.2	SS-RSRQ	3834

A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	3834

A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	3839

A.8.5.2.2.2.1	Test Purpose and Environment	3839

A.8.5.2.2.2.2	Test Parameters	3839

A.8.5.2.2.2.3	Test Requirements	3841

A.8.5.2.3	SS-SINR	3841

A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	3841

A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	3845

A.8.5.2.3.2.1	Test Purpose and Environment	3845

A.8.5.2.3.2.2	Test Parameters	3845

A.8.5.2.3.2.3	Test Requirements	3847

A.9	V2X Tests	3848

A.9.1	V2X Tests in FR1	3848

A.9.1.1	Test for V2X UE Transmit Timing	3848

A.9.1.1.1 Test for GNSS as Synchronization Reference Source	3848

A.9.1.1.1.1	Test Purpose and Environment	3848

A.9.1.1.1.2	Test requirements	3848

A.9.1.1.2	Test for SyncRef UE as Synchronization Reference Source	3848

A.9.1.1.2.1	Test Purpose and Environment	3848

A.9.1.1.2.2	Test requirements	3849

A.9.1.1.3	Test for FR1 NR Cell as Synchronization Reference Source	3849

A.9.1.1.3.1	Test Purpose and Environment	3849

A.9.1.1.3.2	Test requirements	3852

A.9.1.2	Test for Initiation/Cease of S-SSB Transmission with V2X Sidelink Communication	3852

A.9.1.2.1	Test for FR1 NR Cell as synchronization reference source without gap under non-DRX	3852

A.9.1.2.1.1	Test Purpose and Environment	3852

A.9.1.2.1.2	Test Requirements	3855

A.9.1.2.2	Test for SyncRef UE as synchronization reference source	3855

A.9.1.2.2.1	Test Purpose and Environment	3855

A.9.1.2.2.2	Test Requirements	3856

A.9.1.2.3	Test for SyncRef UE as synchronization reference source when SL-DRX is used	3857

A.9.1.2.3.1	Test Purpose and Environment	3857

A.9.1.2.3.2	Test Requirements	3859

A.9.1.3	 Test for V2X Synchronization Reference Selection/Reselection	3859

A.9.1.3.1	 Test for GNSS configured as the highest priority	3859

A.9.1.3.1.1	Test Purpose and Environment	3859

A.9.1.3.1.2	Test Requirements	3861

A.9.1.3.2	 Test for FR1 NR Cell configured as the highest priority	3862

A.9.1.3.2.1	Test Purpose and Environment	3862

A.9.1.3.2.2	Test Requirements	3864

A.9.1.3.3	Test for GNSS configured as the highest priority under SL-DRX	3865

A.9.1.3.3.1	Test Purpose and Environment	3865

A.9.1.3.3.2	Test Requirements	3867

A.9.1.3.4	Test for FR1 NR Cell configured as the highest priority under SL-DRX	3868

A.9.1.3.4.1	Test Purpose and Environment	3868

A.9.1.3.4.2	Test Requirements	3870

A.9.1.4	Test for L1 SL-RSRP Measurement	3871

A.9.1.4.1	Test for V2X UE Autonomous Resource Selection/Reselection	3871

A.9.1.4.1.1	Test Purpose and Environment	3871

A.9.1.4.1.2	Test Requirements	3874

A.9.1.4.2	Test for V2X UE Resource Pre-emption	3874

A.9.1.4.2.1	Test Purpose and Environment	3874

A.9.1.4.2.2	Test Requirements	3878

A.9.1.4.3	 Test for V2X UE Resource Re-evaluation	3878

A.9.1.4.3.1	Test Purpose and Environment	3878

A.9.1.4.3.2	Test Requirements	3885

A.9.1.4.4	Test for V2X UE Autonomous Resource Selection/Reselection with Periodic Sensing	3885

A.9.1.4.4.1	Test Purpose and Environment	3885

A.9.1.4.4.2	Test Requirements	3889

A.9.1.4.5	Test for V2X UE Autonomous Resource Selection/Reselection with Contiguous Sensing	3889

A.9.1.4.5.1	Test Purpose and Environment	3889

A.9.1.4.5.2	Test Requirements	3892

A.9.1.4.6	Test for V2X UE Autonomous Resource Selection/Reselection in SL-DRX	3892

A.9.1.4.6.1	Test Purpose and Environment	3892

A.9.1.4.6.2	Test Requirements	3896

A.9.1.5	Test for Congestion Control Measurement	3896

A.9.1.5.1	Test Purpose and Environment	3896

A.9.1.5.2	Test Requirements	3902

A.9.1.6	Test for Interruption	3902

A.9.1.6.1	Test for Interruption to WAN due to V2X Sidelink Communication	3902

A.9.1.6.1.1	Test Purpose and Environment	3902

A.9.1.6.1.2	Test Requirements	3906

A.9.1.6.2	Test for interruption to WAN at transitions between active and non-active during SL-DRX in asynchronous case	3906

A.9.1.6.2.1	Test Purpose and Environment	3906

A.9.1.6.2.2	Test Requirements	3910

A.9.1.6.3	Test for Interruption at NR Sidelink Diccovery Configuration	3910

A.9.1.6.3.1	Test Purpose and Environment	3910

A.9.1.6.3.2	Test Requirements	3914

A.9.1.7	Selection / Reselection of relay UE	3914

A.9.1.7.1	Test Purpose and Environment	3914

A.9.1.7.2	Test Requirements	3920

A.10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	3917

A.10.1	RRC_CONNECTED state mobility	3917

A.10.1.1	RRC connection mobility control	3917

A.10.1.1.1	Random Access	3917

A.10.1.1.1.1	4-step RA type contention-based random access for NR PSCell with CCA	3917

A.10.1.1.1.1.1	Test Purpose and Environment	3917

A.10.1.1.1.1.2	Test Requirements	3920

A.10.1.1.1.1.2.1	Random Access Preamble Transmission	3920

A.10.1.1.1.1.2.2	Random Access Response Reception	3920

A.10.1.1.1.1.2.3	No Random Access Response Reception	3921

A.10.1.1.1.1.2.4	Receiving an UL grant for msg3 retransmission	3921

A.10.1.1.1.1.2.5	 Contention Resolution Timer expiry	3921

A.10.1.1.1.2	4-step RA type non-contention based random access for NR PSCell with CCA	3922

A.10.1.1.1.2.1	Test Purpose and Environment	3922

A.10.1.1.1.2.2	Test Requirements	3925

A.10.1.1.1.2.2.1	SSB-based Random Access Preamble Transmission	3925

A.10.1.1.1.2.2.2	Random Access Response Reception	3926

A.10.1.1.1.2.2.3	No Random Access Response Reception	3926

A.10.1.1.1.3	2-step RA type contention-based random access for NR PSCell with CCA	3926

A.10.1.1.1.3.1	Test Purpose and Environment	3926

A.10.1.1.1.3.2	Test Requirements	3930

A.10.1.1.1.3.2.1	MsgA Transmission	3930

A.10.1.1.1.3.2.2	MsgB Reception	3930

A.10.1.1.1.3.2.3	No MsgB Reception	3931

A.10.1.1.1.4	2-step RA type non-contention based random access for NR PSCell with CCA	3931

A.10.1.1.1.4.1	Test Purpose and Environment	3931

A.10.1.1.1.4.2	Test Requirements	3935

A.10.1.1.1.4.2.1	MsgA Transmission	3935

A.10.1.1.1.4.2.2	MsgB Reception	3936

A.10.1.1.1.4.2.3	No MsgB Reception	3936

A.10.1.2	Handover with PSCell from EN-DC to EN-DC with known target PSCell using CCA	3936

A.10.1.2.1	Test Purpose and Environment	3936

A.10.1.2.2	Test Requirements	3942

A.10.2	Timing	3943

A.10.2.1	UE transmit timing	3943

A.10.2.1.1	UE Transmit Timing Test with PSCell under DL CCA	3943

A.10.2.1.1.1	Test Purpose and environment	3943

A.10.2.1.1.2	Test requirements	3946

A.10.2.2	UE timing advance	3947

A.10.2.2.1	UE Timing Advance Adjustment Accuracy with PSCell under DL CCA	3947

A.10.2.2.1.1	Test Purpose and Environment	3947

A.10.2.2.1.2	Test Parameters	3947

A.10.2.2.1.3	Test Requirements	3951

A.10.3	Signalling characteristics	3951

A.10.3.1	Radio link monitoring	3951

A.10.3.1.1	Introduction	3951

A.10.3.1.2	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	3952

A.10.3.1.2.1	Test purpose and environment	3952

A.10.3.1.2.2	Test requirements	3956

A.10.3.1.3	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	3957

A.10.3.1.3.1	Test purpose and environment	3957

A.10.3.1.3.2	Test requirements	3962

A.10.3.1.4	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode	3962

A.10.3.1.4.1	Test purpose and environment	3962

A.10.3.1.4.2	Test requirements	3962

A.10.3.1.5	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode	3962

A.10.3.1.5.1	Test purpose and environment	3962

A.10.3.1.5.2	Test requirements	3962

A.10.3.2	Interruption	3962

A.10.3.2.1	E-UTRAN – NR interruptions during SCell operations with CCA	3962

A.10.3.2.1.1	Test Purpose and Environment	3962

A.10.3.2.1.2	Test Requirements	3967

A.10.3.3	SCell activation and deactivation delay	3967

A.10.3.3.1	SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle	3967

A.10.3.3.1.1	Test Purpose and Environment	3967

A.10.3.3.1.2	Test Requirements	3972

A.10.3.3.2 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle	3972

A.10.3.3.2.1	Test Purpose and Environment	3972

A.10.3.3.2.2	Test Requirements	3973

A.10.3.3.3	SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA	3973

A.10.3.3.3.1	Test Purpose and Environment	3973

A.10.3.3.3.2	Test Requirements	3974

A.10.3.4	Beam failure detection and link recovery procedures	3974

A.10.3.4.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	3974

A.10.3.4.1.1	Test Purpose and Environment	3974

A.10.3.4.1.2	Test Requirements	3979

A.10.3.4.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	3979

A.10.3.4.2.1	Test Purpose and Environment	3979

A.10.3.4.2.2	Test Requirements	3985

A.10.3.5	Active BWP switching	3985

A.10.3.5.1	UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA in EN-DC	3985

A.10.3.5.1.2	Test Requirements	3990

A.10.3.5.2	DCI-based and Timer-based Active BWP Switch	3991

A.10.3.5.2.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	3991

A.10.3.5.2.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	3994

A.10.3.5.3	RRC-based Active BWP Switch	3998

A.10.3.5.3.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	3998

A.10.3.6	PSCell addition and release delay	4002

A.10.3.6.1	Addition and Release Delay of known NR PSCell on the carrier under CCA	4002

A.10.3.6.1.1	Test purpose and environment	4002

A.10.3.6.1.2	Test Requirements	4007

A.10.3.7	Void	4008

A.10.4	Measurement procedure	4008

A.10.4.1	Intra-frequency measurements	4008

A.10.4.1.1	Event-triggered reporting tests on PSCC without gaps under non-DRX	4008

A.10.4.1.1.1	Test purpose and environment	4008

A.10.4.1.1.2	Test parameters	4008

A.10.4.1.1.3	Test Requirements	4008

A.10.4.1.2	Event-triggered reporting tests on PSCC without gaps under DRX	4009

A.10.4.1.2.1	Test purpose and environment	4009

A.10.4.1.2.2	Test parameters	4009

A.10.4.1.2.3	Test Requirements	4009

A.10.4.1.3	Event-triggered reporting tests on PSCC with per-UE gaps under non-DRX	4009

A.10.4.1.3.1	Test purpose and environment	4009

A.10.4.1.3.2	Test parameters	4009

A.10.4.1.3.3	Test Requirements	4009

A.10.4.1.4	Event-triggered reporting tests on PSCC with per-UE gaps under DRX	4010

A.10.4.1.4.1	Test purpose and environment	4010

A.10.4.1.4.2	Test parameters	4010

A.10.4.1.4.3	Test Requirements	4010

A.10.4.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	4010

A.10.4.1.5.1	Test purpose and environment	4010

A.10.4.1.5.2	Test parameters	4010

A.10.4.1.5.3	Test Requirements	4010

A.10.4.1.6	Event-triggered reporting tests on SCC without gaps under DRX	4011

A.10.4.1.6.1	Test purpose and environment	4011

A.10.4.1.6.2	Test parameters	4011

A.10.4.1.6.3	Test Requirements	4011

A.10.4.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	4011

A.10.4.1.7.1	Test purpose and environment	4011

A.10.4.1.7.2	Test parameters	4011

A.10.4.1.7.3	Test Requirements	4011

A.10.4.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	4012

A.10.4.1.8.1	Test purpose and environment	4012

A.10.4.1.8.2	Test parameters	4012

A.10.4.1.8.3	Test Requirements	4012

A.10.4.1.9	RSSI measurement reporting on PSCC	4012

A.10.4.1.9.1	Test purpose and environment	4012

A.10.4.1.9.2	Test parameters	4012

A.10.4.1.10	Channel occupancy measurement reporting on PSCC	4013

A.10.4.1.10.1	Test purpose and environment	4013

A.10.4.1.10.2	Test parameters	4013

A.10.4.1.11	RSSI measurement reporting on SCC	4013

A.10.4.1.11.1	Test purpose and environment	4013

A.10.4.1.11.2	Test parameters	4013

A.10.4.1.12	Channel occupancy measurement reporting on SCC	4014

A.10.4.1.12.1	Test purpose and environment	4014

A.10.4.1.12.2	Test parameters	4014

A.10.4.2	Inter-frequency measurements	4014

A.10.4.2.1	RSSI measurement reporting	4014

A.10.4.2.1.1	Test purpose and environment	4014

A.10.4.2.1.2	Test parameters	4014

A.10.4.2.2	Channel occupancy measurement reporting	4015

A.10.4.2.2.1	Test purpose and environment	4015

A.10.4.2.2.2	Test parameters	4015

A.10.4.2.3	EN-DC event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used	4015

A.10.4.2.3.1	Test Purpose and Environment	4015

A.10.4.2.3.2	Test Requirements	4019

A.10.4.2.4	EN-DC event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used	4020

A.10.4.2.4.1	Test Purpose and Environment	4020

A.10.4.2.4.2	Test Requirements	4024

A.10.4.2.5	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used	4025

A.10.4.2.5.1	Test Purpose and Environment	4025

A.10.4.2.5.2	Test Requirements	4029

A.10.4.2.6	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used	4030

A.10.4.2.6.1	Test Purpose and Environment	4030

A.10.4.2.6.2	Test Requirements	4034

A.10.4.2.7	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	4035

A.10.4.2.7.1	Test Purpose and Environment	4035

A.10.4.2.7.2	Test Requirements	4041

A.10.4.2.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	4041

A.10.4.2.8.1	Test Purpose and Environment	4041

A.10.4.2.8.2	Test Requirements	4047

A.10.4.2.9	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	4048

A.10.4.2.9.1	Test Purpose and Environment	4048

A.10.4.2.9.2	Test Requirements	4053

A.10.4.2.10	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	4053

A.10.4.2.10.1	Test Purpose and Environment	4053

A.10.4.2.10.2	Test Requirements	4059

A.10.4.3	L1-RSRP measurements for beam reporting	4060

A.10.4.3.1	SSB based L1-RSRP measurement on PSCC when DRX is not used	4060

A.10.4.3.1.1	Test Purpose and Environment	4060

A.10.4.3.1.2	Test parameters	4060

A.10.4.3.1.3	Test Requirements	4062

A.10.4.3.2	SSB based L1-RSRP measurement on PSCC when DRX is used	4063

A.10.4.3.2.1	Test Purpose and Environment	4063

A.10.4.3.2.2	Test parameters	4063

A.10.4.3.2.3	Test Requirements	4065

A.10.4.3.3	SSB based L1-RSRP measurement on SCC when DRX is not used	4066

A.10.4.3.3.1	Test Purpose and Environment	4066

A.10.4.3.3.2	Test parameters	4066

A.10.4.3.3.3	Test Requirements	4069

A.10.4.3.4	SSB based L1-RSRP measurement on SCC when DRX is used	4070

A.10.4.3.4.1	Test Purpose and Environment	4070

A.10.4.3.4.2	Test parameters	4070

A.10.4.3.4.3	Test Requirements	4073

A.10.4.4	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	4074

A.10.4.4.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4074

A.10.4.4.1.1	Test Purpose and Environment	4074

A.10.4.4.1.2	Test Requirements	4078

A.10.4.4.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4079

A.10.4.4.2.1	Test Purpose and Environment	4079

A.10.4.4.2.2	Test Requirements	4084

A.10.4.4.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4085

A.10.4.4.3.1	Test Purpose and Environment	4085

A.10.4.4.3.2	Test Requirements	4089

A.10.4.4.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4090

A.10.4.4.4.1	Test Purpose and Environment	4090

A.10.4.4.4.2	Test Requirements	4095

A.10.5	Measurement performance	4096

A.10.5.1	SS-RSRP	4096

A.10.5.1.1	Intra-frequency measurement accuracy on a CCA serving cell	4096

A.10.5.1.1.1	Test Purpose and Environment	4096

A.10.5.1.1.2	Test parameters	4096

A.10.5.1.1.3	Test Requirements	4099

A.10.5.1.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4099

A.10.5.1.2.1	Test Purpose and Environment	4099

A.10.5.1.2.2	Test parameters	4099

A.10.5.1.2.3	Test Requirements	4102

A.10.5.2	SS-RSRQ	4102

A.10.5.2.1	Intra-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4102

A.10.5.2.1.1	Test Purpose and Environment	4102

A.10.5.2.1.2	Test Parameters	4102

A.10.5.2.1.3	Test Requirements	4105

A.10.5.2.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4105

A.10.5.2.2.1	Test Purpose and Environment	4105

A.10.5.2.2.2	Test Parameters	4105

A.10.5.2.2.3	Test Requirements	4108

A.10.5.3	SS-SINR	4108

A.10.5.3.1	Intra-frequency measurement accuracy on PSCC	4108

A.10.5.3.1.1	Test Purpose and Environment	4108

A.10.5.3.1.2	Test Parameters	4108

A.10.5.3.1.3	Test Requirements	4111

A.10.5.3.2	Inter-frequency measurement accuracy on PSCC	4111

A.10.5.3.2.1	Test Purpose and Environment	4111

A.10.5.3.2.2	Test Parameters	4111

A.10.5.3.2.3	Test Requirements	4114

A.10.5.3.3	Intra-frequency measurement accuracy on SCC	4114

A.10.5.3.3.1	Test Purpose and Environment	4114

A.10.5.3.3.2	Test Parameters	4114

A.10.5.3.3.3	Test Requirements	4117

A.10.5.4	L1-RSRP measurement for beam reporting with CCA serving cell	4117

A.10.5.4.1	SSB based L1-RSRP measurement	4117

A.10.5.4.1.1	Test Purpose and Environment	4117

A.10.5.4.1.2	Test parameters	4118

A.10.5.4.1.3	Test Requirements	4120

A.10.5.5	RSSI	4120

A.10.5.5.1 	RSSI measurement accuracy on PSCC with CCA	4120

A.10.5.5.1.1	Test Purpose and Environment	4120

A.10.5.5.1.2	Test parameters	4120

A.10.5.5.1.3	Test Requirements	4123

A.10.5.5.2 	RSSI measurement accuracy on SCC with CCA	4123

A.10.5.5.2.1	Test Purpose and Environment	4123

A.10.5.5.2.2	Test parameters	4123

A.10.5.5.2.3	Test Requirements	4126

A.10.5.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4126

A.10.5.5.3.1	Test Purpose and Environment	4126

A.10.5.5.3.2	Test parameters	4126

A.10.5.5.3.3	Test Requirements	4129

A.10.5.6	Channel occupancy	4129

A.10.5.6.1 	Channel occupancy measurement accuracy on PSCC with CCA	4129

A.10.5.6.1.1	Test Purpose and Environment	4129

A.10.5.6.1.2	Test parameters	4129

A.10.5.6.1.3	Test Requirements	4133

A.10.5.6.2 	Channel occupancy measurement accuracy on SCC with CCA	4133

A.10.5.6.2.1	Test Purpose and Environment	4133

A.10.5.6.2.2	Test parameters	4133

A.10.5.6.2.3	Test Requirements	4136

A.10.5.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4136

A.10.5.6.3.1	Test Purpose and Environment	4136

A.10.5.6.3.2	Test parameters	4136

A.10.5.6.3.3	Test Requirements	4140

A.11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	4143

A.11.1	RRC_IDLE state mobility	4143

A.11.1.1	Cell re-selection with both source and target NR carrier frequencies under CCA	4143

A.11.1.1.1	Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell	4143

A.11.1.1.1.1	Test Purpose and Environment	4143

A.11.1.1.1.2	Test Parameters	4143

A.11.1.1.1.3	Test Requirements	4146

A.11.1.1.2	Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell	4146

A.11.1.1.2.1	Test Purpose and Environment	4146

A.11.1.1.2.2	Test Parameters	4146

A.11.1.1.2.3	Test Requirements	4150

A.11.1.2	Cell re-selection to NR with source NR carrier frequency under CCA	4150

A.11.1.2.1	Cell reselection to FR1 inter-frequency NR case when serving cell is subject to CCA	4150

A.11.1.2.1.1	Test Purpose and Environment	4150

A.11.1.2.1.2	Test Parameters	4151

A.11.1.2.1.3	Test Requirements	4157

A.11.1.3	Cell re-selection from NR carrier with target NR carrier frequency under CCA	4158

A.11.1.3.1	Cell reselection to FR1 inter-frequency NR case when target cell is subject to CCA	4158

A.11.1.3.1.1	Test Purpose and Environment	4158

A.11.1.3.1.2	Test Parameters	4158

A.11.1.3.1.3	Test Requirements	4164

A.11.1.4	Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA	4165

A.11.1.4.1	Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA	4165

A.11.1.4.1.1	Test Purpose and Environment	4165

A.11.1.4.1.2	Test Parameters	4165

A.11.1.4.1.3	Test Requirements	4168

A.11.1.4.2	Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA	4169

A.11.1.4.2.1	Test Purpose and Environment	4169

A.11.1.4.2.2	Test Requirements	4172

A.11.2	RRC_CONNECTED state mobility	4173

A.11.2.1	Handover	4173

A.11.2.1.1	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell	4173

A.11.2.1.1.1	Test Purpose and Environment	4173

A.11.2.1.1.2	Test Parameters	4173

A.11.2.1.1.3 Test Requirements	4176

A.11.2.1.2	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	4176

A.11.2.1.2.1	Test Purpose and Environment	4176

A.11.2.1.2.2	Test Parameters	4176

A.11.2.1.2.3	Test Requirements	4179

A.11.2.1.3	Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	4179

A.11.2.1.3.1	Test Purpose and Environment	4179

A.11.2.1.3.2	Test Parameters	4179

A.11.2.1.3.3 Test Requirements	4182

A.11.2.1.4	Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell	4182

A.11.2.1.4.1	Test Purpose and Environment	4182

A.11.2.1.4.2	Test Parameters	4182

A.11.2.1.4.3 Test Requirements	4186

A.11.2.1.5	Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell	4187

A.11.2.1.5.1	Test Purpose and Environment	4187

A.11.2.1.5.2	Test Parameters	4187

A.11.2.1.5.3 Test Requirements	4190

A.11.2.1.6	Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell	4191

A.11.2.1.6.1	Test Purpose and Environment	4191

A.11.2.1.6.2	Test Parameters	4191

A.11.2.1.6.3 Test Requirements	4194

A.11.2.1.7	 SA NR FR1 carrier under CCA - E-UTRAN handover with known target cell	4195

A.11.2.1.7.1	Test Purpose and Environment	4195

A.11.2.1.7.2	Test Requirements	4201

A.11.2.1.8	SA NR FR1 carrier under CCA - E-UTRAN handover with unknown target cell	4201

A.11.2.1.8.1	Test Purpose and Environment	4201

A.11.2.1.8.2	Test Requirements	4206

A.11.2.1.9	Handover with PSCell from NR SA to EN-DC with known target PSCell using CCA	4206

A.11.2.1.9.1	Test Purpose and Environment	4206

A.11.2.1.9.2	Test Requirements	4215

A.11.2.2	RRC connection mobility control	4216

A.11.2.2.1	RRC re-establishment	4216

A.11.2.2.1.1	Intra-frequency RRC Re-establishment with CCA in FR1	4216

A.11.2.2.1.2	Inter-frequency RRC Re-establishment with CCA in FR1	4219

A.11.2.2.1.4	Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA	4226

A.11.2.2.2	Random Access	4231

A.11.2.2.2.1	4-step RA type contention-based random access for NR PCell with CCA	4231

A.11.2.2.2.1.1	Test Purpose and Environment	4231

A.11.2.2.2.1.2	Test Requirements	4234

A.11.2.2.2.1.2.1	Random Access Preamble Transmission	4234

A.11.2.2.2.1.2.2	Random Access Response Reception	4234

A.11.2.2.2.1.2.3	No Random Access Response Reception	4235

A.11.2.2.2.1.2.4	Receiving an UL grant for msg3 retransmission	4235

A.11.2.2.2.1.2.5	Reception of an Incorrect Message over Temporary C-RNTI	4235

A.11.2.2.2.1.2.6	Reception of a Correct Message over Temporary C-RNTI	4236

A.11.2.2.2.1.2.7	Contention Resolution Timer expiry	4236

A.11.2.2.2.2	4-step RA type non-contention based random access for NR PSCell with CCA	4236

A.11.2.2.2.2.1	Test Purpose and Environment	4236

A.11.2.2.2.2.2	Test Requirements	4239

A.11.2.2.2.2.2.1	SSB-based Random Access Preamble Transmission	4239

A.11.2.2.2.2.2.2	Random Access Response Reception	4240

A.11.2.2.2.2.2.3	No Random Access Response Reception	4240

A.11.2.2.2.3	2-step RA type contention-based random access for NR PCell with CCA	4240

A.11.2.2.2.3.1	Test Purpose and Environment	4240

A.11.2.2.2.3.2	Test Requirements	4244

A.11.2.2.2.3.2.1	MsgA Transmission	4244

A.11.2.2.2.3.2.2	MsgB Reception	4244

A.11.2.2.2.3.2.3	No MsgB Reception	4245

A.11.2.2.2.4	2-step RA type non-contention-based random access for NR PCell with CCA	4245

A.11.2.2.2.4.1	Test Purpose and Environment	4245

A.11.2.2.2.4.2	Test Requirements	4249

A.11.2.2.2.4.2.1	MsgA Transmission	4249

A.11.2.2.2.4.2.2	MsgB Reception	4250

A.11.2.2.2.4.2.3	No MsgB Reception	4250

A.11.2.2.3	RRC connection release with redirection	4251

A.11.2.2.3.1	Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA	4251

A.11.2.2.3.2	Redirection from NR FR1 carrier without CCA to NR FR1 carrier with CCA	4255

A.11.3	Timing	4260

A.11.3.1	UE transmit timing	4260

A.11.3.1.1	UE Transmit Timing Test with PCell under DL CCA	4260

A.11.3.1.1.1	Test Purpose and environment	4260

A.11.3.1.1.2	Test requirements	4263

A.11.3.2	UE timing advance	4264

A.11.3.2.1	UE Timing Advance Adjustment Accuracy with PCell under DL CCA	4264

A.11.3.2.1.1	Test Purpose and Environment	4264

A.11.3.2.1.2	Test Parameters	4264

A.11.3.2.1.3	Test Requirements	4268

A.11.4	Signalling characteristics	4268

A.11.4.1	Radio link monitoring	4268

A.11.4.1.1	Introduction	4268

A.11.4.1.2	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	4269

A.11.4.1.2.1	Test purpose and environment	4269

A.11.4.1.2.2	Test requirements	4273

A.11.4.1.3	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	4273

A.11.4.1.3.1	Test purpose and environment	4273

A.11.4.1.3.2	Test requirements	4279

A.11.4.1.4	Void	4279

A.11.4.1.4.1	Void	4279

A.11.4.1.4.2	Void	4279

A.11.4.1.5	Void	4279

A.11.4.1.5.1	Void	4279

A.11.4.1.5.2	Void	4279

A.11.4.2	Interruption	4280

A.11.4.2.1	NR interruptions during Scell operations with CCA on PCell and SCell	4280

A.11.4.2.1.1 Test Purpose and Environment	4280

A.11.4.2.1.2	Test Requirements	4284

A.11.4.3	SCell activation and deactivation delay	4284

A.11.4.3.1	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle	4284

A.11.4.3.1.1	Test Purpose and Environment	4284

A.11.4.3.1.2	Test Requirements	4289

A.11.4.3.2	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 640 ms SCell measurement cycle	4289

A.11.4.3.2.1	Test Purpose and Environment	4289

A.11.4.3.2.2	Test Requirements	4290

A.11.4.3.3	SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA	4290

A.11.4.3.3.1	Test Purpose and Environment	4290

A.11.4.3.3.2	Test Requirements	4291

A.11.4.4	Beam failure detection and link recovery procedures	4291

A.11.4.4.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	4291

A.11.4.4.1.1	Test Purpose and Environment	4291

A.11.4.4.1.2	Test Requirements	4296

A.11.4.4.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	4296

A.11.4.4.2.1	Test Purpose and Environment	4296

A.11.4.4.2.2	Test Requirements	4302

A.11.4.5	Active BWP switching	4302

A.11.4.5.1	UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA	4302

A.11.4.5.1.1	Test Purpose and Environment	4302

A.11.4.5.1.2	Test Requirements	4306

A.11.4.5.2	DCI-based and Timer-based Active BWP Switch	4307

A.11.4.5.2.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA	4307

A.11.4.5.2.2	NR FR1 DL active BWP switch with non-DRX in SA	4310

A.11.4.5.3	RRC-based Active BWP Switch	4313

A.11.4.5.3.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	4313

A.11.4.6	Void	4316

A.11.5	Measurement procedure	4316

A.11.5.1	Intra-frequency measurements	4316

A.11.5.1.1	Event-triggered reporting tests on PCC without gaps under non-DRX	4316

A.11.5.1.1.1	Test purpose and environment	4316

A.11.5.1.1.2	Test parameters	4316

A.11.5.1.1.3	Test Requirements	4317

A.11.5.1.2	Event-triggered reporting tests on PCC without gaps under DRX	4317

A.11.5.1.2.1	Test purpose and environment	4317

A.11.5.1.2.2	Test parameters	4317

A.11.5.1.2.3	Test Requirements	4317

A.11.5.1.3	Event-triggered reporting tests on PCC with per-UE gaps under non-DRX	4318

A.11.5.1.3.1	Test purpose and environment	4318

A.11.5.1.3.2	Test parameters	4318

A.11.5.1.3.3	Test Requirements	4318

A.11.5.1.4	Event-triggered reporting tests on PCC with per-UE gaps under DRX	4318

A.11.5.1.4.1	Test purpose and environment	4318

A.11.5.1.4.2	Test parameters	4318

A.11.5.1.4.3	Test Requirements	4318

A.11.5.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	4319

A.11.5.1.5.1	Test purpose and environment	4319

A.11.5.1.5.2	Test parameters	4319

A.11.5.1.6	Event-triggered reporting tests on SCC without gaps under DRX	4319

A.11.5.1.6.1	Test purpose and environment	4319

A.11.5.1.6.2	Test parameters	4319

A.11.5.1.6.3	Test Requirements	4319

A.11.5.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	4320

A.11.5.1.7.1	Test purpose and environment	4320

A.11.5.1.7.2	Test parameters	4320

A.11.5.1.7.3	Test Requirements	4320

A.11.5.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	4320

A.11.5.1.8.1	Test purpose and environment	4320

A.11.5.1.8.2	Test parameters	4320

A.11.5.1.8.3	Test Requirements	4320

A.11.5.1.9	RSSI measurement reporting on PCC	4321

A.11.5.1.9.1	Test purpose and environment	4321

A.11.5.1.9.2	Test parameters	4321

A.11.5.1.10	Channel occupancy measurement reporting on PCC	4321

A.11.5.1.10.1	Test purpose and environment	4321

A.11.5.1.10.2	Test parameters	4321

A.11.5.1.11	RSSI measurement reporting on SCC	4321

A.11.5.1.11.1	Test purpose and environment	4321

A.11.5.1.11.2	Test parameters	4322

A.11.5.1.12	Channel occupancy measurement reporting on SCC	4322

A.11.5.1.12.1	Test purpose and environment	4322

A.11.5.1.12.2	Test parameters	4322

A.11.5.2	Inter-frequency measurements	4322

A.11.5.2.1	RSSI measurement reporting	4322

A.11.5.2.1.1	Test purpose and environment	4322

A.11.5.2.1.2	Test parameters	4323

A.11.5.2.2	Channel occupancy measurement reporting	4323

A.11.5.2.2.1	Test purpose and environment	4323

A.11.5.2.2.2	Test parameters	4323

A.11.5.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	4323

A.11.5.2.3.1	Test Purpose and Environment	4323

A.11.5.2.3.2	Test Requirements	4327

A.11.5.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	4328

A.11.5.2.4.1	Test Purpose and Environment	4328

A.11.5.2.4.2	Test Requirements	4333

A.11.5.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	4333

A.11.5.2.5.1	Test Purpose and Environment	4333

A.11.5.2.5.2	Test Requirements	4337

A.11.5.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	4338

A.11.5.2.6.1	Test Purpose and Environment	4338

A.11.5.2.6.2	Test Requirements	4343

A.11.5.2.7	Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4343

A.11.5.2.7.1	Test Purpose and Environment	4343

A.11.5.2.7.2	Test Requirements	4347

A.11.5.2.8	Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4348

A.11.5.2.8.1	Test Purpose and Environment	4348

A.11.5.2.8.2	Test Requirements	4353

A.11.5.2.9	Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4353

A.11.5.2.9.1	Test Purpose and Environment	4353

A.11.5.2.9.2	Test Requirements	4358

A.11.5.2.10	Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4359

A.11.5.2.10.1	Test Purpose and Environment	4359

A.11.5.2.10.2	Test Requirements	4364

A.11.5.3	Inter-RAT E-UTRAN measurements	4365

A.11.5.4	L1-RSRP measurements for beam reporting	4365

A.11.5.4.1	SSB based L1-RSRP measurement when DRX is not used	4365

A.11.5.4.1.1	Test Purpose and Environment	4365

A.11.5.4.1.2	Test parameters	4365

A.11.5.4.1.3	Test Requirements	4367

A.11.5.4.2	SSB based L1-RSRP measurement when DRX is used	4367

A.11.5.4.2.1	Test Purpose and Environment	4367

A.11.5.4.2.2	Test parameters	4368

A.11.5.4.2.3	Test Requirements	4370

A.11.5.4.3	SSB based L1-RSRP measurement on SCC when DRX is not used	4370

A.11.5.4.3.1	Test Purpose and Environment	4370

A.11.5.4.3.2	Test parameters	4371

A.11.5.4.3.3	Test Requirements	4374

A.11.5.4.4	SSB based L1-RSRP measurement on SCC when DRX is used	4375

A.11.5.4.4.1	Test Purpose and Environment	4375

A.11.5.4.4.2	Test parameters	4375

A.11.5.4.4.3	Test Requirements	4378

A.11.6	Measurement performance	4379

A.11.6.1	SS-RSRP	4379

A.11.6.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	4379

A.11.6.1.1.1	Test Purpose and Environment	4379

A.11.6.1.1.2	Test parameters	4379

A.11.6.1.1.3	Test Requirements	4381

A.11.6.1.2	Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	4381

A.11.6.1.2.1	Test Purpose and Environment	4381

A.11.6.1.2.2	Test parameters	4381

A.11.6.1.2.3	Test Requirements	4383

A.11.6.2	SS-RSRQ	4383

A.11.6.2.1	Intra-frequency measurement accuracy	4383

A.11.6.2.1.1	Test Purpose and Environment	4383

A.11.6.2.1.2	Test Parameters	4383

A.11.6.2.1.3	Test Requirements	4386

A.11.6.2.2	Inter-frequency measurement accuracy	4386

A.11.6.2.2.1	Test Purpose and Environment	4386

A.11.6.2.2.2	Test Parameters	4386

A.11.6.2.2.3	Test Requirements	4389

A.11.6.2.3	Intra-frequency measurement accuracy on SCC	4389

A.11.6.2.3.1	Test Purpose and Environment	4389

A.11.6.2.3.2	Test Parameters	4389

A.11.6.2.3.3	Test Requirements	4392

A.11.6.2.4	Inter-frequency measurement accuracy	4392

A.11.6.2.4.1	Test Purpose and Environment	4392

A.11.6.2.4.2	Test Parameters	4392

A.11.6.2.4.3	Test Requirements	4401

A.11.6.3	SS-SINR	4401

A.11.6.3.1	Intra-frequency measurement accuracy	4401

A.11.6.3.1.1	Test Purpose and Environment	4401

A.11.6.3.1.2	Test Parameters	4401

A.11.6.3.1.3	Test Requirements	4404

A.11.6.3.2	Inter-frequency measurement accuracy	4404

A.11.6.3.2.1	Test Purpose and Environment	4404

A.11.6.3.2.2	Test Parameters	4404

A.11.6.3.2.3	Test Requirements	4407

A.11.6.3.3	Intra-frequency measurement accuracy on SCC	4407

A.11.6.3.3.1	Test Purpose and Environment	4407

A.11.6.3.3.2	Test Parameters	4407

A.11.6.3.3.3	Test Requirements	4410

A.11.6.3.4	Inter-frequency measurement accuracy	4410

A.11.6.3.4.1	Test Purpose and Environment	4410

A.11.6.3.4.2	Test Parameters	4410

A.11.6.3.4.3	Test Requirements	4418

A.11.6.4	L1-RSRP measurement for beam reporting with CCA serving cell	4418

A.11.6.4.1	SSB based L1-RSRP measurement	4418

A.11.6.4.1.1	Test Purpose and Environment	4418

A.11.6.4.1.2	Test parameters	4419

A.11.6.4.1.3	Test Requirements	4422

A.11.6.5	RSSI	4422

A.11.6.5.1 	Intra-frequency RSSI measurement accuracy on PCC with CCA	4422

A.11.6.5.1.1	Test Purpose and Environment	4422

A.11.6.5.1.2	Test parameters	4422

A.11.6.5.1.3	Test Requirements	4425

A.11.6.5.2 	Intra-frequency RSSI measurement accuracy on SCC with CCA	4425

A.11.6.5.2.1	Test Purpose and Environment	4425

A.11.6.5.2.2	Test parameters	4425

A.11.6.5.2.3	Test Requirements	4428

A.11.6.5.3	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4428

A.11.6.5.3.1	Test Purpose and Environment	4428

A.11.6.5.3.2	Test parameters	4428

A.11.6.5.3.3	Test Requirements	4431

A.11.6.6	Channel occupancy	4431

A.11.6.6.1	Intra-frequency channel occupancy measurement accuracy on PCC with CCA	4431

A.11.6.6.1.1	Test Purpose and Environment	4431

A.11.6.6.1.2	Test parameters	4431

A.11.6.6.1.3	Test Requirements	4434

A.11.6.6.2	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	4434

A.11.6.6.2.1	Test Purpose and Environment	4434

A.11.6.6.2.2	Test parameters	4434

A.11.6.6.2.3	Test Requirements	4437

A.11.6.6.3	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4437

A.11.6.6.3.1	Test Purpose and Environment	4437

A.11.6.6.3.2	Test parameters	4437

A.11.6.6.3.3	Test Requirements	4440

A.11.6.7	E-UTRAN RSRP	4440

A.11.6.8	E-UTRAN RSRQ	4440

A.11.6.9	E-UTRAN SINR	4440

A.12	E-UTRA Standalone Tests with at Least One NR Cell under CCA	4440

A.12.1	RRC_IDLE state mobility	4440

A.12.1.1	Inter-RAT cell re-selection to NR on a carrier frequency with CCA	4440

A.12.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA	4440

A.12.1.1.1.1	Test Purpose and Environment	4440

A.12.1.1.1.2	Test Requirements	4445

A.12.2	RRC_CONNECTED state mobility	4446

A.12.2.1	Handover	4446

A.12.2.1.1	E-UTRAN - NR with CCA handover	4446

A.12.2.1.1.1	Test Purpose and Environment	4446

A.12.2.1.1.2	Test Requirements	4451

A.12.3	Signalling characteristics	4452

A.12.3.1	Interruptions	4452

A.12.4	Measurement procedure	4452

A.12.4.1	E-UTRANNR inter-RAT SFTD measurements	4452

A.12.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay with NR under CCA in non-DRX	4452

A.12.4.1.1.1	Test Purpose and Environment	4452

A.12.4.1.1.2	Test Requirements	4456

A.12.4.2	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	4456

A.12.4.2.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4456

A.12.4.2.1.1	Test Purpose and Environment	4456

A.12.4.2.1.2	Test Requirements	4460

A.12.4.2.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4461

A.12.4.2.2.1	Test Purpose and Environment	4461

A.12.4.2.2.2	Test Requirements	4465

A.12.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4465

A.12.4.2.3.1	Test Purpose and Environment	4465

A.12.4.2.3.2	Test Requirements	4469

A.12.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4470

A.12.4.2.4.1	Test Purpose and Environment	4470

A.12.4.2.4.2	Test Requirements	4474

A.12.4.2.5	RSSI measurement reporting	4474

A.12.4.2.5.1	Test purpose and environment	4474

A.12.4.2.5.2	Test parameters	4474

A.12.4.2.6	Channel occupancy measurement reporting	4475

A.12.4.2.6.1	Test purpose and environment	4475

A.12.4.2.6.2	Test parameters	4475

A.12.5	Measurement performance	4475

A.12.5.1	E-UTRANNR SFTD	4475

A.12.5.1.1	Inter-RAT SFTD accuracy with NR target cell under CCA	4475

A.12.5.1.1.1	Test Purpose	4475

A.12.5.1.1.2	Test Environment	4476

A.12.5.1.1.3	Test Requirements	4481

A.12.5.2	E-UTRANNR SS-RSRP	4481

A.12.5.3	E-UTRANNR SS-RSRQ	4481

A.12.5.4	E-UTRANNR SS-SINR	4481

A.12.5.5	E-UTRANNR RSSI	4481

A.12.5.6	E-UTRANNR channel occupancy	4481

A.13	NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	4482

A.13.1	Timing	4482

A.13.1.1	UE transmit timing	4482

A.13.1.2	Timing advance	4482

A.13.2	Signalling characteristics	4482

A.13.2.1	Interruption	4482

A.13.2.1.1	NR interruptions during SCell operations with CCA on SCell	4482

A.13.2.1.1.1	Test Purpose and Environment	4482

A.13.2.1.1.2	Test Requirements	4486

A.13.2.2	SCell activation and deactivation delay	4486

A.13.2.2.1	SCell Activation and Deactivation of known SCell under CCA, 160 ms SCell measurement cycle	4486

A.13.2.2.1.1	Test Purpose and Environment	4486

A.13.2.2.1.2	Test Requirements	4491

A.13.2.2.2 SCell Activation and Deactivation of known SCell under CCA, 640 ms SCell measurement cycle	4491

A.13.2.2.2.1	Test Purpose and Environment	4491

A.13.2.2.2.2	Test Requirements	4492

A.13.2.2.3	SCell Activation and Deactivation of unknown SCell under CCA	4492

A.13.2.2.3.1	Test Purpose and Environment	4492

A.13.2.2.3.2	Test Requirements	4493

A.13.2.3	Void	4493

A.13.3	Measurement procedure	4493

A.13.3.1	Intra-frequency measurements	4493

A.13.3.1.1	Event-triggered reporting tests on SCC without gaps under non-DRX	4493

A.13.3.1.1.1	Test purpose and environment	4493

A.13.3.1.1.2	Test parameters	4493

A.13.3.1.1.3	Test Requirements	4498

A.13.3.1.2	Event-triggered reporting tests on SCC without gaps under DRX	4498

A.13.3.1.2.1	Test purpose and environment	4498

A.13.3.1.2.2	Test parameters	4498

A.13.3.1.2.3	Test Requirements	4503

A.13.3.1.3	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	4503

A.13.3.1.3.1	Test purpose and environment	4503

A.13.3.1.3.2	Test parameters	4503

A.13.3.1.3.3	Test Requirements	4508

A.13.3.1.4	Event-triggered reporting tests on SCC with per-UE gaps under DRX	4508

A.13.3.1.4.1	Test purpose and environment	4508

A.13.3.1.4.2	Test parameters	4508

A.13.3.1.4.3	Test Requirements	4513

A.13.3.1.5	RSSI measurement reporting on SCC	4513

A.13.3.1.5.1	Test purpose and environment	4513

A.13.3.1.5.2	Test parameters	4513

A.13.3.1.6	Channel occupancy measurement reporting on SCC	4514

A.13.3.1.6.1	Test purpose and environment	4514

A.13.3.1.6.2	Test parameters	4515

A.13.3.2	Inter-frequency measurements	4515

A.13.3.2.1	RSSI measurement reporting	4515

A.13.3.2.1.1	Test purpose and environment	4515

A.13.3.2.1.2	Test parameters	4515

A.13.3.2.2	Channel occupancy measurement reporting	4516

A.13.3.2.2.1	Test purpose and environment	4516

A.13.3.2.2.2	Test parameters	4516

A.13.3.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	4516

A.13.3.2.3.1	Test Purpose and Environment	4516

A.13.3.2.3.2	Test Requirements	4521

A.13.3.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	4521

A.13.3.2.4.1	Test Purpose and Environment	4521

A.13.3.2.4.2	Test Requirements	4526

A.13.3.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	4527

A.13.3.2.5.1	Test Purpose and Environment	4527

A.13.3.2.5.2	Test Requirements	4532

A.13.3.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	4532

A.13.3.2.6.1	Test Purpose and Environment	4532

A.13.3.2.6.2	Test Requirements	4538

A.13.3.3	L1-RSRP measurements for beam reporting	4539

A.13.3.3.1	SSB based L1-RSRP measurement when DRX is not used	4539

A.13.3.3.1.1	Test Purpose and Environment	4539

A.13.3.3.1.2	Test parameters	4539

A.13.3.3.1.3	Test Requirements	4543

A.13.3.3.2	SSB based L1-RSRP measurement when DRX is used	4543

A.13.3.3.2.1	Test Purpose and Environment	4543

A.13.3.3.2.2	Test parameters	4543

A.13.3.3.2.3	Test Requirements	4547

A.13.4	Measurement performance	4547

A.13.4.1	SS-RSRP	4547

A.13.4.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	4547

A.13.4.1.1.1	Test Purpose and Environment	4547

A.13.4.1.1.2	Test parameters	4547

A.13.4.1.1.3	Test Requirements	4549

A.13.4.2	SS-RSRQ	4549

A.13.4.2.1	Intra-frequency measurement accuracy on SCC	4549

A.13.4.2.1.1	Test Purpose and Environment	4549

A.13.4.2.1.2	Test Parameters	4549

A.13.4.2.1.3	Test Requirements	4556

A.13.4.3	SS-SINR	4557

A.13.4.3.1	Intra-frequency measurement accuracy on SCC	4557

A.13.4.3.1.1	Test Purpose and Environment	4557

A.13.4.3.1.2	Test Parameters	4557

A.13.4.3.1.3	Test Requirements	4564

A.13.4.4	L1-RSRP measurement for beam reporting with CCA serving cell	4564

A.13.4.4.1	SSB based L1-RSRP measurement	4564

A.13.4.4.1.1	Test Purpose and Environment	4564

A.13.4.4.1.2	Test parameters	4564

A.13.4.4.1.3	Test Requirements	4568

A.13.4.5	RSSI	4568

A.13.4.5.1 	Intra-frequency RSSI measurement accuracy on a carrier with CCA	4568

A.13.4.5.1.1	Test Purpose and Environment	4568

A.13.4.5.1.2	Test parameters	4568

A.13.4.5.1.3	Test Requirements	4571

A.13.4.5.2	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4571

A.13.4.5.2.1	Test Purpose and Environment	4571

A.13.4.5.2.2	Test parameters	4571

A.13.4.5.2.3	Test Requirements	4575

A.13.4.6	Channel occupancy	4575

A.13.4.6.1	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	4575

A.13.4.6.1.1	Test Purpose and Environment	4575

A.13.4.6.1.2	Test parameters	4575

A.13.4.6.1.3	Test Requirements	4579

A.13.4.6.2	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4579

A.13.4.6.2.1	Test Purpose and Environment	4579

A.13.4.6.2.2	Test parameters	4579

A.13.4.6.2.3	Test Requirements	4583

A.14	NR standalone tests for Satellite access	4583

A.14.1	RRC_IDLE state mobility	4583

A.14.1.1	Cell reselection to FR1 intra-frequency NR case	4583

A.14.1.1.1	Test Purpose and Environment	4583

A.14.1.1.2	Test Parameters	4583

A.14.1.1.3	Test Requirements	4585

A.14.1.2	Cell reselection to FR1 intra-frequency NR cell for UE configured with [capability for enhanced requirements]	4586

A.14.1.2.1	Test Purpose and Environment	4586

A.14.1.2.2	Test Parameters	4586

A.14.1.2.3	Test Requirements	4590

A.14.1.3	Time-based cell reselection to FR1 intra-frequency NR cell	4590

A.14.1.3.1	Test Purpose and Environment	4590

A.14.1.3.2	Test Parameters	4590

A.14.1.3.3	Test Requirements	4592

A.14.1.4	Location-based cell reselection to FR1 intra-frequency NR cell	4593

A.14.1.4.1	Test Purpose and Environment	4593

A.14.1.4.2	Test Parameters	4593

A.14.1.4.3	Test Requirements	4595

A.14.1.5	Cell reselection to FR1 inter-frequency NR case	4596

A.14.1.5.1	Test Purpose and Environment	4596

A.14.1.5.2	Test Parameters	4596

A.14.1.5.3	Test Requirements	4598

A.14.1.6	Cell reselection to FR1 inter-frequency NR cell for UE configured with [capability for enhanced requirements]	4599

A.14.1.6.1	Test Purpose and Environment	4599

A.14.1.6.2	Test Parameters	4599

A.14.1.6.3	Test Requirements	4601

A.14.1.7	Time-based Cell reselection to FR1 inter-frequency NR satellite access case	4602

A.14.1.7.1	Test Purpose and Environment	4602

A.14.1.7.2	Test Parameters	4602

A.14.1.7.3	Test Requirements	4603

A.14.1.8	Location-based Cell reselection to FR1 inter-frequency NR satellite access case	4604

A.14.1.8.1	Test Purpose and Environment	4604

A.14.1.8.2	Test Parameters	4604

A.14.1.8.3	Test Requirements	4606

A.14.1.9	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	4607

A.14.1.9.1	Test Purpose and Environment	4607

A.14.1.9.2	Test Parameters	4607

A.14.1.9.3	Test Requirements	4610

A.14.1.10	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	4610

A.14.1.10.1	Test Purpose and Environment	4610

A.14.1.10.2	Test Parameters	4610

A.14.1.10.3	Test Requirements	4612

A.14.2	RRC_CONNECTED state mobility	4613

A.14.2.1	Handover	4613

A.14.2.1.1	Intra-frequency SAN Handover from FR1 to FR1	4613

A.14.2.1.1.1	Test Purpose and Environment	4613

A.14.2.1.1.2	Test Parameters	4613

A.14.2.1.1.3	Test Requirements	4615

A.14.2.1.2	Inter-frequency SAN Handover from FR1 to FR1	4616

A.14.2.1.2.1	Test Purpose and Environment	4616

A.14.2.1.2.2	Test Parameters	4616

A.14.2.1.2.3	Test Requirements	4618

A.14.2.1.3	Intra-frequency SAN time-based conditional Handover from FR1 to FR1	4618

A.14.2.1.3.1	Test Purpose and Environment	4618

A.14.2.1.3.2	Test Parameters	4618

A.14.2.1.3.3	Test Requirements	4621

A.14.2.1.4	Inter-frequency SAN time-based conditional Handover from FR1 to FR1	4621

A.14.2.1.4.1	Test Purpose and Environment	4621

A.14.2.1.4.2	Test Parameters	4621

A.14.2.1.4.3	Test Requirements	4623

A.14.2.1.5	Intra-frequency SAN distance-based conditional Handover from FR1 to FR1	4623

A.14.2.1.5.1	Test Purpose and Environment	4623

A.14.2.1.5.2	Test Parameters	4624

A.14.2.1.5.3	Test Requirements	4627

A.14.2.1.6	Inter-frequency SAN distance-based conditional Handover from FR1 to FR1	4627

A.14.2.1.6.1	Test Purpose and Environment	4627

A.14.2.1.6.2	Test Parameters	4627

A.14.2.1.6.3	Test Requirements	4631

A.14.2.2	RRC Connection Mobility Control	4631

A.14.2.2.1	SA: RRC Re-establishment for SAN	4631

A.14.2.2.1.1	Intra-frequency RRC Re-establishment in FR1	4631

A.14.2.2.1.2	Inter-frequency RRC Re-establishment in FR1	4634

A.14.2.2.2	Random Access	4637

A.14.2.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	4637

A.14.2.2.2.1.1	Test Purpose and Environment	4637

A.14.2.2.2.1.2	Test Requirements	4640

A.14.2.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	4641

A.14.2.2.2.2.1	Test Purpose and Environment	4641

A.14.2.2.2.2.2	Test Requirements	4645

A.14.2.2.3	RRC Connection Release with Redirection	4646

A.14.2.2.3.1	Redirection from NR in FR1 to NR in FR1	4646

A.14.2.2.3.1.1	Test Purpose and Environment	4646

A.14.2.2.3.1.2	Test Parameters	4646

A.14.2.2.3.1.3	Test Requirements	4649

A.14.3	Timing for Satellite Access	4649

A.14.3.1	UE transmit timing for Satellite Access	4649

A.14.3.1.1	NR UE Transmit Timing Test for FR1	4649

A.14.3.1.1.1	Test Purpose and environment	4649

A.14.3.1.1.2	Test requirements	4651

A.14.3.2	Timing advance for satellite access	4652

A.14.3.2.1	SA FR1 timing advance adjustment accuracy	4652

A.14.3.2.1.1	Test Purpose and Environment	4652

A.14.3.2.1.2	Test Parameters	4652

A.14.3.2.1.3	Test Requirements	4655

A.14.4	Signalling characteristics	4655

A.14.4.1	Radio link Monitoring	4655

A.14.4.1.1	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode	4655

A.14.4.1.1.1	Test Purpose and Environment	4655

A.14.4.1.1.2	Test Requirements	4660

A.14.4.1.2	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode	4660

A.14.4.1.2.1	Test Purpose and Environment	4660

A.14.4.1.2.2	Test Requirements	4666

A.14.4.1.3	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode	4666

A.14.4.1.3.1	Test Purpose and Environment	4666

A.14.4.1.3.2	Test Requirements	4670

A.14.4.1.4	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode	4670

A.14.4.1.4.1	Test Purpose and Environment	4670

A.14.4.1.4.2	Test Requirements	4675

A.14.4.1.5	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode	4675

A.14.4.1.5.1	Test Purpose and Environment	4675

A.14.4.1.5.2	Test Requirements	4680

A.14.4.1.6	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode	4680

A.14.4.1.6.1	Test Purpose and Environment	4680

A.14.4.1.6.2	Test Requirements	4685

A.14.4.1.7	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode	4685

A.14.4.1.7.1	Test Purpose and Environment	4685

A.14.4.1.7.2	Test Requirements	4690

A.14.4.1.8	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode	4690

A.14.4.1.8.1	Test Purpose and Environment	4690

A.14.4.1.8.2	Test Requirements	4695

A.14.4.2	Beam Failure Detection and Link recovery procedures for satellite access	4695

A.14.4.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in non-DRX mode	4695

A.14.4.2.1.1	Test Purpose and Environment	4695

A.14.4.2.1.2	Test Requirements	4701

A.14.4.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in DRX mode	4701

A.14.4.2.2.1	Test Purpose and Environment	4701

A.14.4.2.2.2	Test Requirements	4706

A.14.4.2.3	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in non-DRX mode	4706

A.14.4.2.3.1	Test Purpose and Environment	4706

A.14.4.2.3.2	Test Requirements	4712

A.14.4.2.4	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in DRX mode	4712

A.14.4.2.4.1	Test Purpose and Environment	4712

A.14.4.2.4.2	Test Requirements	4717

A.14.4.2.5	Beam Failure Detection and Link Recovery Test for FR1 SCell for satellite access configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	4717

A.14.4.2.5.1	Test Purpose and Environment	4717

A.14.4.2.5.2	Test Requirements	4722

A.14.4.2.6	Beam Failure Detection and Link Recovery Test for FR1 SCell for satellite access configured with CSI-RS-based BFD and SSB-based LR in DRX mode	4722

A.14.4.2.6.1	Test Purpose and Environment	4722

A.14.4.2.6.2	Test Requirements	4727

A.14.4.3	Active BWP switch for satellite access	4727

A.14.4.3.1	DCI-based and Timer-based Active BWP Switch	4727

A.14.4.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA	4727

A.14.4.3.1.1.1	Test Purpose and Environment	4727

A.14.4.3.1.1.2	Test Requirements	4732

A.14.4.3.2	RRC-based Active BWP Switch	4732

A.14.4.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	4732

A.14.4.3.2.1.1	Test Purpose and Environment	4732

A.14.4.3.2.1.2	Test Requirements	4736

A.14.4.4	UE specific CBW change for satellite access	4736

A.14.4.4.1	UE specific CBW change on PCell in FR1 in non-DRX	4736

A.14.4.4.1.1	Test Purpose and Environment	4736

A.14.4.4.1.2	Test Requirements	4740

A.14.4.5	Pathloss reference signal switching delay	4740

A.14.4.5.1	MAC-CE based pathloss reference signal switch delay	4740

A.14.4.5.1.1	Test Purpose and Environment	4740

A.14.4.5.1.2	Test Requirements	4742

A.14.5	Measurement procedure	4743

A.14.5.1	Intra-frequency Measurements	4743

A.14.5.1.1	SA event triggered reporting tests without gap under non-DRX	4743

A.14.5.1.1.1	Test purpose and Environment	4743

A.14.5.1.1.2	Test parameters	4743

A.14.5.1.1.3	Test Requirements	4745

A.14.5.1.2	SA event triggered reporting tests without gap under DRX	4746

A.14.5.1.2.1	Test purpose and Environment	4746

A.14.5.1.2.2	Test parameters	4746

A.14.5.1.2.3	Test Requirements	4748

A.14.5.1.3	SA event triggered reporting tests without gap under non-DRX with SSB index reading	4749

A.14.5.1.3.1	Test purpose and Environment	4749

A.14.5.1.3.2	Test parameters	4749

A.14.5.1.3.3	Test Requirements	4751

A.14.5.1.4	SA event triggered reporting tests with single measurement gap under non-DRX for satellite access	4752

A.14.5.1.4.1	Test purpose and Environment	4752

A.14.5.1.4.2	Test parameters	4752

A.14.5.1.4.3	Test Requirements	4754

A.14.5.1.5	SA event triggered reporting tests with FNO concurrent gaps under DRX for satellite access	4755

A.14.5.1.5.1	Test purpose and Environment	4755

A.14.5.1.5.2	Test parameters	4755

A.15.5.1.5.3	Test Requirements	4757

A.14.5.1.6	SA event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for satellite access	4758

A.14.5.1.6.1	Test purpose and Environment	4758

A.14.5.1.6.2	Test parameters	4758

A.14.5.1.6.3	Test Requirements	4760

A.14.5.2	Inter-frequency Measurements	4761

A.14.5.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with single gap for satellite access	4761

A.14.5.2.1.1	Test Purpose and Environment	4761

A.14.5.2.1.2	Test Requirements	4765

A.14.5.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used with single gap for satellite access	4765

A.14.5.2.2.1	Test Purpose and Environment	4765

A.14.5.2.2.2	Test Requirements	4769

A.14.5.2.3	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used with single gap for satellite access	4769

A.14.5.2.3.1	Test Purpose and Environment	4769

A.14.5.2.3.2	Test Requirements	4773

A.14.5.2.4	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in fully non-overlapped for satellite access	4773

A.14.5.2.4.1	Test Purpose and Environment	4773

A.14.5.2.4.2	Test Requirements	4777

A.14.5.2.5	void	4777

A.14.5.2.5.1	void	4777

A.14.5.2.5.2	void	4777

A.14.5.2.6	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in partially partial overalpping for satellite access	4777

A.14.5.2.6.1	Test Purpose and Environment	4777

A.14.5.2.6.2	Test Requirements	4781

A.14.5.2.7	Event triggered reporting test without gap under non-DRX	4781

A.14.5.2.7.1	Test purpose and Environment	4781

A.14.5.2.7.2	Test parameters	4781

A.14.5.2.7.3	Test Requirements	4783

A.14.5.2.8	Event triggered reporting tests without gap under DRX	4784

A.14.5.2.8.1	Test purpose and Environment	4784

A.14.5.2.8.2	Test parameters	4784

A.14.5.2.8.3	Test Requirements	4786

A.14.5.3	L1-RSRP measurement for beam reporting for satellite access	4787

A.14.5.3.1	SSB based L1-RSRP measurement for satellite access when DRX is not used	4787

A.14.5.3.1.1	Test Purpose and Environment	4787

A.14.5.3.1.2	Test parameters	4787

A.14.5.3.1.3	Test Requirements	4790

A.14.5.3.2	SSB based L1-RSRP measurement for satellite access when DRX is used	4790

A.14.5.3.2.1	Test Purpose and Environment	4790

A.14.5.3.2.2	Test parameters	4790

A.14.5.3.2.3	Test Requirements	4794

A.14.5.3.3	CSI-RS based L1-RSRP measurement for satellite access when DRX is not used	4794

A.14.5.3.3.1	Test Purpose and Environment	4794

A.14.5.3.3.2	Test parameters	4794

A.14.5.3.3.3	Test Requirements	4798

A.14.5.3.4	CSI-RS based L1-RSRP measurement for satellite access when DRX is used	4798

A.14.5.3.4.1	Test Purpose and Environment	4798

A.14.5.3.4.2	Test parameters	4799

A.14.5.3.4.3	Test Requirements	4802

A.14.6	Measurement Performance requirements	4802

A.14.6.1	 SS-RSRP for SAN	4802

A.14.6.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	4802

A.14.6.1.1.1	Test Purpose and Environment	4802

A.14.6.1.1.2	Test parameters	4802

A.14.6.1.1.3	Test Requirements	4806

A.14.6.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	4806

A.14.6.1.2.1	Test Purpose and Environment	4806

A.14.6.1.2.2	Test parameters	4806

A.14.6.1.2.3	Test Requirements	4809

A.14.6.2	SS-RSRQ	4809

A.14.6.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for satellite access	4809

A.14.6.2.1.1	Test Purpose and Environment	4809

A.14.6.2.1.2	Test Parameters	4809

A.14.6.2.1.3	Test Requirements	4812

A.14.6.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for satellite access	4812

A.14.6.2.2.1	Test Purpose and Environment	4812

A.14.6.2.2.2	Test Parameters	4812

A.14.6.2.2.3	Test Requirements	4815

A.14.6.3	SS-SINR	4815

A.14.6.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	4815

A.14.6.3.1.1	Test Purpose and Environment	4815

A.14.6.3.1.2	Test Parameters	4815

A.14.6.3.1.3	Test Requirements	4819

A.14.6.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	4819

A.14.6.3.2.1	Test Purpose and Environment	4819

A.14.6.3.2.2	Test Parameters	4819

A.14.6.3.2.3	Test Requirements	4823

A.14.6.4	L1-RSRP measurement for beam reporting	4823

A.14.6.4.1	SSB based L1-RSRP measurement	4823

A.14.6.4.1.1	Test Purpose and Environment	4823

A.14.6.4.1.2	Test parameters	4823

A.14.6.4.1.3	Test Requirements	4826

A.14.6.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	4827

A.14.6.4.2.1	Test Purpose and Environment	4827

A.14.6.4.2.2	Test parameters	4827

A.14.6.4.2.3	Test Requirements	4830

A.15	NR standalone tests with one or more NR cells in FR2-2	4831

A.15.1	SA: RRC_IDLE state mobility	4831

A.15.1.1	Cell re-selection to NR	4831

A.15.1.1.1	Cell reselection to FR2-2 intra-frequency NR case	4831

A.15.1.1.1.1	Test Purpose and Environment	4831

A.15.1.1.1.2	Test Parameters	4831

A.15.1.1.1.3	Test Requirements	4835

A.15.1.2	Cell reselection to FR2-2 inter-frequency NR case	4835

A.15.1.2.1	Test Purpose and Environment	4835

A.15.1.2.2	Test Parameters	4835

A.15.1.2.3	Test Requirements	4839

A.15.1.3	Cell reselection to FR2-2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	4839

A.15.1.3.1	Test Purpose and Environment	4839

A.15.1.3.2	Test Parameters	4839

A.15.1.3.3	Test Requirements	4842

A.15.1.4	Cell reselection to FR2-2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	4842

A.15.1.4.1	Test Purpose and Environment	4842

A.15.1.4.2	Test Parameters	4842

A.15.1.4.3	Test Requirements	4845

A.15.1.5	Cell reselection to FR2-2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	4845

A.15.1.5.1	Test Purpose and Environment	4845

A.15.1.5.2	Test Parameters	4845

A.15.1.5.3	Test Requirements	4849

A.15.1.6	Cell reselection to FR2-2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	4849

A.15.1.6.1	Test Purpose and Environment	4849

A.15.1.6.2	Test Parameters	4849

A.15.1.6.3	Test Requirements	4852

A.15.2	Signaling characteristics	4852

A.15.2.1	SCell Activation and Deactivation Delay	4852

A.15.2.1.1	SCell Activation and deactivation for SCell in FR2-2 intra-band in non-DRX	4852

A.15.2.1.1.1	Test Purpose and Environment	4852

A.15.2.1.1.2	Test Requirements	4855

A.15.2.1.2	SCell Activation and deactivation for FR1+FR2-2 inter-band with target SCell in FR2-2	4855

A.15.2.1.2.1	Test Purpose and Environment	4855

A.15.2.1.2.2	Test Requirements	4860

A.15.2.1.3	SCell Activation and deactivation for SCell in FR2-2 inter-band in non-DRX	4861

A.15.2.1.3.1	Test Purpose and Environment	4861

A.15.2.1.3.2	Test Requirements	4864

A.15.2.1.4	Direct SCell activation at SCell addition of known SCell in FR2-2	4865

A.15.2.1.4.1	Test Purpose and Environment	4865

A.15.2.1.4.2	Test Requirements	4868

A.15.2.1.5	Direct SCell activation at handover with known SCell in FR2-2	4869

A.15.2.1.5.1	Test Purpose and Environment	4869

A.15.2.1.5.2	Test Requirements	4873

A.15.3	RRC_CONNECTED state mobility	4874

A.15.3.1	Handover	4874

A.15.3.1.1	Intra-frequency handover from FR2-2 carrier with CCA to FR2-2 carrier with CCA; unknown target cell	4874

A.15.3.1.1.1	Test Purpose and Environment	4874

A.15.3.1.1.2	Test Parameters	4874

A.15.3.1.1.3	Test Requirements	4878

A.15.3.1.2	Inter-frequency handover from FR1 to FR2-2 carrier with CCA; unknown target cell	4878

A.15.3.1.2.1	Test Purpose and Environment	4878

A.15.3.1.2.2	Test Parameters	4878

A.15.3.1.2.3	Test Requirements	4882

A.15.4	Measurement procedure	4883

A.15.4.1	Intra-frequency Measurements	4883

A.15.4.1.1	SA event triggered reporting test without gap under non-DRX for FR2-2 with CCA	4883

A.15.4.1.1.1	Test purpose and Environment	4883

A.15.4.1.1.2	Test Requirements	4885

A.15.4.2	Inter-frequency Measurements	4886

A.15.4.2.1	SA event triggered reporting tests for FR2-2 with CCA without SSB time index detection when DRX is not used (PCell in FR2-2)	4886

A.15.4.2.1.1	Test Purpose and Environment	4886

A.15.4.2.1.2	Test Requirements	4889

A.16	NR standalone tests with all NR cells in FR1 for RedCap	4890

A.16.1	SA: RRC_IDLE state mobility for RedCap	4890

A.16.1.1	Cell re-selection to NR	4890

A.16.1.1.1	Cell reselection to FR1 intra-frequency NR case for 1 Rx UE	4890

A.16.1.1.1.1	Test Purpose and Environment	4890

A.16.1.1.1.2	Test Parameters	4890

A.16.1.1.1.3	Test Requirements	4894

A.16.1.1.2	Cell reselection to FR1 intra-frequency NR case for 2 Rx UE	4894

A.16.1.1.2.1	Test Purpose and Environment	4894

A.16.1.1.2.2	Test Parameters	4894

A.16.1.1.2.3	Test Requirements	4898

A.16.1.1.3	Cell reselection to FR1 inter-frequency NR case for 1 Rx UE	4898

A.16.1.1.3.1	Test Purpose and Environment	4898

A.16.1.1.3.2	Test Parameters	4898

A.16.1.1.3.3	Test Requirements	4903

A.16.1.1.4	Cell reselection to FR1 inter-frequency NR case for 2 Rx UE	4903

A.16.1.1.4.1	Test Purpose and Environment	4903

A.16.1.1.4.2	Test Parameters	4903

A.16.1.1.4.3	Test Requirements	4908

A.16.1.1.5	Cell reselection to FR1 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	4908

A.16.1.1.5.1	Test Purpose and Environment	4908

A.16.1.1.5.2	Test Parameters	4908

A.16.1.1.5.3	Test Requirements	4913

A.16.1.1.6	Cell reselection to FR1 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	4913

A.16.1.1.6.1	Test Purpose and Environment	4913

A.16.1.1.6.2	Test Parameters	4913

A.16.1.1.6.3	Test Requirements	4918

A.16.1.1.7	Cell reselection to FR1 inter-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	4918

A.16.1.1.7.1	Test Purpose and Environment	4918

A.16.1.1.7.2	Test Parameters	4918

A.16.1.1.7.3	Test Requirements	4923

A.16.1.1.8	Cell reselection to FR1 inter-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	4923

A.16.1.1.8.1	Test Purpose and Environment	4923

A.16.1.1.8.2	Test Parameters	4924

A.16.1.1.8.3	Test Requirements	4928

A.16.1.2	Inter-RAT E-UTRAN cell re-selection for RedCap	4929

A.16.1.2.1	Cell reselection to higher priority E-UTRAN for 1RX	4929

A.16.1.2.1.1	Test Purpose and Environment	4929

A.16.1.2.1.2	Test Parameters	4929

A.16.1.2.1.3	Test Requirements	4932

A.16.1.2.2	Cell reselection to higher priority E-UTRAN for 2RX	4933

A.16.1.2.2.1	Test Purpose and Environment	4933

A.16.1.2.2.2	Test Parameters	4933

A.16.1.2.2.3	Test Requirements	4936

A.16.1.2.3.1	Test Purpose and Environment	4937

A. 16.1.2.3.2	Test Parameters	4937

A.16.1.2.3.3	Test Requirements	4940

A.16.1.2.4.1	Test Purpose and Environment	4941

A.16.1.2.4.2	Test Parameters	4941

A.16.1.3.1.3	Test Requirements	4944

A.16.1.2.5	Cell reselection to lower priority E-UTRAN for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	4945

A.16.1.2.5.1	Test Purpose and Environment	4945

A.16.1.2.5.2	Test Parameters	4945

A.16.1.2.5.3	Test Requirements	4948

A.16.1.2.6	Cell reselection to lower priority E-UTRAN for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	4949

A.16.1.2.6.1	Test Purpose and Environment	4949

A.16.1.2.6.2	Test Parameters	4949

A.16.1.2.6.3	Test Requirements	4952

A.16.2	SA: RRC_INACTIVE state mobility for RedCap	4953

A.16.2.1	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	4953

A.16.2.1.1	NR UE CG-SDT Test in FR1 for 1Rx RedCap UE	4953

A.16.2.1.1.1	Test purpose and Environment	4953

A.16.2.1.1.2	Test Parameters	4954

A.16.2.1.1.3	Test requirements	4957

A.16.2.1.2	NR UE CG-SDT Test in FR1 for 2Rx RedCap UE	4957

A.16.2.1.2.1	Test purpose and Environment	4957

A.16.2.1.2.2	Test Parameters	4958

A.16.2.1.2.3	Test requirements	4961

A.16.3	RRC_CONNECTED state mobility for RedCap	4961

A.16.3.1	Handover	4961

A.16.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell for 1 Rx UE	4961

A.16.3.1.1.1	Test Purpose and Environment	4961

A.16.3.1.1.2	Test Parameters	4961

A.16.3.1.1.3	Test Requirements	4965

A.16.3.1.2	Intra-frequency handover from FR1 to FR1; known target cell for 2 Rx UE	4965

A.16.3.1.2.1	Test Purpose and Environment	4965

A.16.3.1.2.2	Test Parameters	4965

A.16.3.1.2.3	Test Requirements	4969

A.16.3.1.3	Intra-frequency handover from FR1 to FR1; unknown target cell for 1 Rx UE	4969

A.16.3.1.3.1	Test Purpose and Environment	4969

A.16.3.1.3.2	Test Parameters	4969

A.16.3.1.3.3	Test Requirements	4973

A.16.3.1.4	Intra-frequency handover from FR1 to FR1; unknown target cell for 2 Rx UE	4973

A.16.3.1.4.1	Test Purpose and Environment	4973

A.16.3.1.4.2	Test Parameters	4974

A.16.3.1.5	Inter-frequency handover from FR1 to FR1; unknown target cell for 1 Rx UE	4977

A.16.3.1.5.1	Test Purpose and Environment	4977

A.16.3.1.5.2	Test Parameters	4977

A.16.3.1.5.3	Test Requirements	4981

A.16.3.1.6	Inter-frequency handover from FR1 to FR1; unknown target cell for 2 Rx UE	4981

A.16.3.1.6.1	Test Purpose and Environment	4981

A.16.3.1.6.2	Test Parameters	4981

A.16.3.1.6.3	Test Requirements	4985

A.16.3.1.7	SA NR - E-UTRAN handover for 1Rx UE	4985

A.16.3.1.7.1	Test Purpose and Environment	4985

A.16.3.1.7.2	Test Requirements	4991

A.16.3.1.8	 SA NR - E-UTRAN handover for 2Rx UE	4991

A.16.3.1.8.1	Test Purpose and Environment	4991

A.16.3.1.8.2	Test Requirements	4997

A.16.3.1.9	SA NR - E-UTRAN handover with unknown target cell for 1 Rx UE	4997

A.16.3.1.9.1	Test Purpose and Environment	4997

A.16.3.1.9.2	Test Requirements	5004

A.16.3.1.10	SA NR - E-UTRAN handover with unknown target cell for 2 Rx UE	5004

A.16.3.1.10.1	Test Purpose and Environment	5004

A.16.3.1.10.2	Test Requirements	5010

A.16.3.2	RRC Connection Mobility Control	5010

A.16.3.2.1	SA: RRC Re-establishment	5010

A.16.3.2.1.1	Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE	5010

A.16.3.2.1.2	Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE	5014

A.16.3.2.1.3	Inter-frequency RRC Re-establishment in FR1 for 1 Rx UE	5018

A.16.3.2.1.4	Inter-frequency RRC Re-establishment in FR1 for 2 Rx UE	5023

A.16.3.2.1.5	Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE without serving cell timing	5028

A.16.3.2.1.6	Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE without serving cell timing	5032

A.16.3.2.2	Random Access	5036

A.16.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	5036

A.16.3.2.2.2	4-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	5042

A.16.3.2.2.3	4-step RA type non-contention based random access test in FR1 for NR standalone for 1 Rx UE	5047

A.16.3.2.2.5	2-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	5057

A.16.3.2.2.6	2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	5061

A.16.3.2.2.7	2-step RA type non-contention based test in FR1 for NR standalone for 1 RX UE	5065

A.16.3.2.2.8	2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE	5069

A.16.3.2.3	SA: RRC Connection Release with Redirection	5073

A.16.3.2.3.1	Redirection from NR in FR1 to NR in FR1 for 1 Rx UE	5073

A.16.3.2.3.2	Redirection from NR in FR1 to NR in FR1 for 2 Rx UE	5077

A.16.3.2.3.3	Redirection from NR in FR1 to E-UTRAN for 1 Rx UE	5081

A.16.3.2.3.4	Redirection from NR in FR1 to E-UTRAN for 2 Rx UE	5088

A.16.4	Timing for RedCap	5095

A.16.4.1	UE transmit timing	5095

A.16.4.1.1	NR UE Transmit Timing Test for FR1 for 1Rx RedCap UE	5095

A.16.4.1.1.1	Test Purpose and environment	5095

A.16.4.1.1.2	Test requirements	5097

A.16.4.1.2	NR UE Transmit Timing Test for FR1 for 2Rx RedCap UE	5098

A.16.4.1.2.1	Test Purpose and environment	5098

A.16.4.1.2.2	Test requirements	5100

A.16.4.2	UE timer accuracy	5100

A.16.4.3	Timing advance	5100

A.16.4.3.1	SA FR1 timing advance adjustment accuracy for 1 Rx UE	5100

A.16.4.3.1.1	Test Purpose and Environment	5100

A.16.4.3.1.2	Test Parameters	5100

A.16.4.3.1.3	Test Requirements	5104

A.16.4.3.2	SA FR1 timing advance adjustment accuracy for 2 Rx UE	5104

A.16.4.3.2.1	Test Purpose and Environment	5104

A.16.4.3.2.2	Test Parameters	5105

A.16.4.3.2.3	Test Requirements	5108

A.16.5	Signalling characteristics for RedCap	5109

A.16.5.1	Radio link Monitoring	5109

A.16.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	5109

A.16.5.1.1.1	Test Purpose and Environment	5109

A.16.5.1.1.2	Test Requirements	5114

A.16.5.1.2	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	5114

A.16.5.1.2.1	Test Purpose and Environment	5114

A.16.5.1.2.2	Test Requirements	5120

A.16.5.1.3	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	5120

A.16.5.1.3.1	Test Purpose and Environment	5120

A.16.5.1.3.2	Test Requirements	5126

A.16.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	5126

A.16.5.1.4.1	Test Purpose and Environment	5126

A.16.5.1.4.2	Test Requirements	5132

A.16.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	5132

A.16.5.1.5.1	Test Purpose and Environment	5132

A.16.5.1.5.2	Test Requirements	5138

A.16.5.1.6	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	5138

A.16.5.1.6.1	Test Purpose and Environment	5138

A.16.5.1.6.2	Test Requirements	5143

A.16.5.1.7	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	5143

A.16.5.1.7.1	Test Purpose and Environment	5143

A.16.5.1.7.2	Test Requirements	5148

A.16.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	5148

A.16.5.1.8.1	Test Purpose and Environment	5148

A.16.5.1.8.2	Test Requirements	5154

A.16.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	5154

A.16.5.1.9.1	Test Purpose and Environment	5154

A.16.5.1.9.2	Test Requirements	5160

A.16.5.1.10	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	5160

A.16.5.1.10.1	Test Purpose and Environment	5160

A.16.5.1.10.2	Test Requirements	5166

A.16.5.1.11	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	5166

A.16.5.1.11.1	Test Purpose and Environment	5166

A.16.5.1.11.2	Test Requirements	5172

A.16.5.1.12	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	5172

A.16.5.1.12.1	Test Purpose and Environment	5172

A.16.5.1.12.2	Test Requirements	5177

A.16.5.1.13	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	5177

A.16.5.1.13.1	Test Purpose and Environment	5177

A.16.5.1.13.2	Test Requirements	5183

A.16.5.1.14	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	5183

A.16.5.1.14.1	Test Purpose and Environment	5183

A.16.5.1.14.2	Test Requirements	5189

A.16.5.1.15	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	5189

A.16.5.1.15.1	Test Purpose and Environment	5189

A.16.5.1.15.2	Test Requirements	5195

A.16.5.1.16	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	5195

A.16.5.1.16.1	Test Purpose and Environment	5195

A.16.5.1.16.2	Test Requirements	5201

A.16.5.2	Beam Failure Detection and Link recovery procedures	5201

A.16.5.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 1 Rx UE	5201

A.16.5.2.1.1	Test Purpose and Environment	5201

A.16.5.2.1.2	Test Requirements	5207

A.16.5.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	5207

A.16.5.2.2.1	Test Purpose and Environment	5207

A.16.5.2.2.2	Test Requirements	5213

A.16.5.2.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 1 Rx UE	5213

A.16.5.2.3.1	Test Purpose and Environment	5213

A.16.5.2.3.2	Test Requirements	5220

A.16.5.2.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 2 Rx UE	5220

A.16.5.2.4.1	Test Purpose and Environment	5220

A.16.5.2.4.2	Test Requirements	5226

A.16.5.2.5	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 1 Rx UE	5226

A.16.5.2.5.1	Test Purpose and Environment	5226

A.16.5.2.5.2	Test Requirements	5232

A.16.5.2.6	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE	5232

A.16.5.2.6.1	Test Purpose and Environment	5232

A.16.5.2.6.2	Test Requirements	5238

A.16.5.2.7	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 1 Rx UE	5238

A.16.5.2.7.1	Test Purpose and Environment	5238

A.16.5.2.7.2	Test Requirements	5244

A.16.5.2.8	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE	5244

A.16.5.2.8.1	Test Purpose and Environment	5244

A.16.5.2.8.2	Test Requirements	5250

A.16.5.3	Active BWP switch	5250

A.16.5.3.1	DCI-based and Timer-based Active BWP Switch	5250

A.16.5.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA for 1 Rx UE	5250

A.16.5.3.1.1.1	Test Purpose and Environment	5250

A.16.5.3.1.1.2	Test Requirements	5255

A.16.5.3.1.2	NR FR1 DL active BWP switch with non-DRX in SA for 2 Rx UE	5255

A.16.5.3.1.1.2	Test Purpose and Environment	5255

A.16.5.3.1.2.2	Test Requirements	5260

A.16.5.3.2	RRC-based Active BWP Switch	5260

A.16.5.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA for 1 Rx UE	5260

A.16.5.3.2.1.1	Test Purpose and Environment	5260

A.16.5.3.2.1.2	Test Requirements	5264

A.16.5.3.2.2	NR FR1 DL active BWP switch of Cell with non-DRX in SA for 2 Rx UE	5264

A.16.5.3.2.2.1	Test Purpose and Environment	5264

A.16.5.3.2.2.2	Test Requirements	5268

A.16.5.4	UE specific CBW change	5268

A.16.5.4.1	UE specific CBW change on PCell in FR1 in non-DRX for 1 Rx UE	5268

A.16.5.4.1.1	Test Purpose and Environment	5268

A.16.5.4.1.2	Test Requirements	5272

A.16.5.4.2	UE specific CBW change on PCell in FR1 in non-DRX for 2 Rx UE	5272

A.16.5.4.2.1	Test Purpose and Environment	5272

A.16.5.4.2.2	Test Requirements	5276

A.16.6	Measurement procedure for RedCap	5276

A.16.6.1	Intra-frequency Measurements	5276

A.16.6.1.1	SA event triggered reporting tests without gap under non-DRX for 1 Rx UE	5276

A.16.6.1.1.1	Test purpose and Environment	5276

A.16.6.1.1.2	Test parameters	5276

A.16.6.1.1.3	Test Requirements	5280

A.16.6.1.2	SA event triggered reporting tests without gap under non-DRX for 2 Rx UE	5280

A.16.6.1.2.1	Test purpose and Environment	5280

A.16.6.1.2.2	Test parameters	5280

A.16.6.1.2.3	Test Requirements	5284

A.16.6.1.3	SA event triggered reporting tests without gap under DRX for 1 Rx UE	5284

A.16.6.1.3.1	Test purpose and Environment	5284

A.16.6.1.3.2	Test parameters	5284

A.16.6.1.3.3	Test Requirements	5288

A.16.6.1.4	SA event triggered reporting tests without gap under DRX for 2 Rx UE	5288

A.16.6.1.4.1	Test purpose and Environment	5288

A.16.6.1.4.2	Test parameters	5288

A.16.6.1.4.3	Test Requirements	5292

A.16.6.1.5	SA event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE	5292

A.16.6.1.5.1	Test purpose and Environment	5292

A.16.6.1.5.2	Test parameters	5292

A.16.6.1.5.3	Test Requirements	5296

A.16.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX for 2 Rx UE	5296

A.16.6.1.6.1	Test purpose and Environment	5296

A.16.6.1.6.2	Test parameters	5296

A.16.6.1.6.3	Test Requirements	5300

A.16.6.1.7	SA event triggered reporting tests with per-UE gaps under DRX for 1 Rx UE	5300

A.16.6.1.7.1	Test purpose and Environment	5300

A.16.6.1.7.2	Test parameters	5300

A.16.6.1.7.3	Test Requirements	5304

A.16.6.1.8	SA event triggered reporting tests with per-UE gaps under DRX for 2 Rx UE	5304

A.16.6.1.8.1	Test purpose and Environment	5304

A.16.6.1.8.2	Test parameters	5304

A.16.6.1.8.3	Test Requirements	5308

A.16.6.1.9	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 1 Rx UE	5308

A.16.6.1.9.1	Test purpose and Environment	5308

A.16.6.1.9.2	Test parameters	5308

A.16.6.1.9.3	Test Requirements	5310

A.16.6.1.10	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 2 Rx UE	5311

A.16.6.1.10.1	Test purpose and Environment	5311

A.16.6.1.10.2	Test parameters	5311

A.16.6.1.10.3	Test Requirements	5312

A.16.6.1.11	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 1 Rx UE	5313

A.16.6.1.11.1	Test purpose and Environment	5313

A.16.6.1.11.2	Test parameters	5313

A.16.6.1.11.3	Test Requirements	5315

A.16.6.1.12	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 2 Rx UE	5316

A.16.6.1.12.1	Test purpose and Environment	5316

A.16.6.1.12.2	Test parameters	5316

A.16.6.1.12.3	Test Requirements	5318

A.16.6.2	Inter-frequency Measurements	5319

A.16.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE	5319

A.16.6.2.1.1	Test Purpose and Environment	5319

A.16.6.2.1.2	Test Requirements	5324

A.16.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE	5324

A.16.6.2.2.1	Test Purpose and Environment	5324

A.16.6.2.2.2	Test Requirements	5329

A.16.6.2.3	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE	5329

A.16.6.2.3.1	Test Purpose and Environment	5329

A.16.6.2.3.2	Test Requirements	5332

A.16.6.2.4	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE	5333

A.16.6.2.4.1	Test Purpose and Environment	5333

A.16.6.2.4.2	Test Requirements	5336

A.16.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 1 Rx UE	5337

A.16.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 2 Rx UE	5337

A.16.6.2.7	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 1 Rx UE	5337

A.16.6.2.7.1	Test Purpose and Environment	5337

A.16.6.2.7.2	Test Requirements	5341

A.16.6.2.8	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 2 Rx UE	5342

A.16.6.2.8.1	Test Purpose and Environment	5342

A.16.6.2.8.2	Test Requirements	5346

A.16.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern for 1 Rx UE	5347

A.16.6.2.9.1	Test Purpose and Environment	5347

A.16.6.2.9.2	Test Requirements	5349

A.16.6.2.10	SA event triggered reporting tests with additional mandatory gap pattern for 2 Rx UE	5350

A.16.6.2.10.1	Test Purpose and Environment	5350

A.16.6.2.10.2	Test Requirements	5352

A.16.6.2.11	SA event triggered reporting tests for FR1 when DRX is used for 1 Rx UE	5353

A.16.6.2.11.1	Test Purpose and Environment	5353

A.16.6.2.11.2	Test Requirements	5356

A.16.6.2.12	SA event triggered reporting tests for FR1 when DRX is used for 2 Rx UE	5356

A.16.6.2.12.1	Test Purpose and Environment	5356

A.16.6.2.12.2	Test Requirements	5360

A.16.6.3	Inter-RAT Measurements	5361

A.16.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for 1 Rx UE	5361

A.16.6.3.1.1	Test purpose and Environment	5361

A.16.6.3.1.2	Test Requirements	5366

A.16.6.3.2	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for 2 Rx UE	5366

A.16.6.3.2.1	Test purpose and Environment	5366

A.16.6.3.2.2	Test Requirements	5371

A.16.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for 1 Rx UE	5371

A.16.6.3.3.1	Test purpose and Environment	5371

A.16.6.3.3.2	Test Requirements	5376

A.16.6.3.4	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for 2 Rx UE	5376

A.16.6.3.4.1	Test purpose and Environment	5376

A.16.6.3.4.2	Test Requirements	5381

A.16.6.4	L1-RSRP measurement for beam reporting	5381

A.16.6.4.1	SSB based L1-RSRP measurement when DRX is not used for 1 Rx UE	5381

A.16.6.4.1.1	Test Purpose and Environment	5381

A.16.6.4.1.2	Test parameters	5381

A.16.6.4.1.3	Test Requirements	5384

A.16.6.4.2	SSB based L1-RSRP measurement when DRX is not used for 2 Rx UE	5384

A.16.6.4.2.1	Test Purpose and Environment	5384

A.16.6.4.2.2	Test parameters	5385

A.16.6.4.2.3	Test Requirements	5388

A.16.6.4.3	SSB based L1-RSRP measurement when DRX is used for 1 Rx UE	5388

A.16.6.4.3.1	Test Purpose and Environment	5388

A.16.6.4.3.2	Test parameters	5389

A.16.6.4.3.3	Test Requirements	5392

A.16.6.4.4	SSB based L1-RSRP measurement when DRX is used for 2 Rx UE	5392

A.16.6.4.4.1	Test Purpose and Environment	5392

A.16.6.4.4.2	Test parameters	5393

A.16.6.4.4.3	Test Requirements	5396

A.16.6.4.5	CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE	5396

A.16.6.4.5.1	Test Purpose and Environment	5396

A.16.6.4.5.2	Test parameters	5397

A.16.6.4.5.3	Test Requirements	5400

A.16.6.4.6	CSI-RS based L1-RSRP measurement when DRX is not used for 2 Rx UE	5400

A.16.6.4.6.1	Test Purpose and Environment	5400

A.16.6.4.6.2	Test parameters	5401

A.16.6.4.6.3	Test Requirements	5404

A.16.6.4.7	CSI-RS based L1-RSRP measurement when DRX is used for 1 Rx UE	5404

A.16.6.4.7.1	Test Purpose and Environment	5404

A.16.6.4.7.2	Test parameters	5405

A.16.6.4.7.3	Test Requirements	5408

A.16.6.4.8	CSI-RS based L1-RSRP measurement when DRX is used for 2 Rx UE	5408

A.16.6.4.8.1	Test Purpose and Environment	5408

A.16.6.4.8.2	Test parameters	5409

A.16.6.4.8.3	Test Requirements	5412

A.16.6.7	NR measurements with autonomous gaps	5412

A.16.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1 for 1 Rx UE	5412

A.16.6.7.1.1	Test Purpose and Environment	5412

A.16.6.7.1.2	Test Parameters	5412

A.16.6.7.1.3	Test Requirements	5415

A.16.6.7.2	SA intra-frequency CGI identification of NR neighbor cell in FR1 for 2 Rx UE	5415

A.16.6.7.2.1	Test Purpose and Environment	5415

A.16.6.7.2.2	Test Parameters	5415

A.16.6.7.2.3	Test Requirements	5418

A.16.6.7.3	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 1 Rx UE	5418

A.16.6.7.3.1	Test Purpose and Environment	5418

A.16.6.7.3.2	Test Requirements	5425

A.16.6.7.4	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 2 Rx UE	5425

A.16.6.7.4.1	Test Purpose and Environment	5425

A.16.6.7.4.2	Test Requirements	5432

A.16.7	Measurement Performance requirements for RedCap	5432

A.16.7.1	SS-RSRP	5432

A.16.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5432

A.16.7.1.1.1	Test Purpose and Environment	5432

A.16.7.1.1.2	Test parameters	5432

A.16.7.1.1.3	Test Requirements	5437

A.16.7.1.2	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 2Rx UE	5437

A.16.7.1.2.1	Test Purpose and Environment	5437

A.16.7.1.2.2	Test parameters	5437

A.16.7.1.2.3	Test Requirements	5442

A.16.7.1.3	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5442

A.16.7.1.3.1	Test Purpose and Environment	5442

A.16.7.1.3.2	Test parameters	5442

A.16.7.1.3.3	Test Requirements	5446

A.16.7.1.4	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5446

A.16.7.1.4.1	Test Purpose and Environment	5446

A.16.7.1.4.2	Test parameters	5447

A.16.7.1.4.3	Test Requirements	5451

A.16.7.2	SS-RSRQ	5451

A.16.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5451

A.16.7.2.1.1	Test Purpose and Environment	5451

A.16.7.2.1.2	Test Parameters	5452

A.16.7.2.1.3	Test Requirements	5457

A.16.7.2.2	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5457

A.16.7.2.2.1	Test Purpose and Environment	5457

A.16.7.2.2.2	Test Parameters	5457

A.16.7.2.2.3	Test Requirements	5462

A.16.7.2.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5462

A.16.7.2.3.1	Test Purpose and Environment	5462

A.16.7.2.3.2	Test parameters	5462

A.16.7.2.3.3	Test Requirements	5467

A.16.7.2.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5467

A.16.7.2.4.1	Test Purpose and Environment	5467

A.16.7.2.4.2	Test parameters	5467

A.16.7.2.4.3	Test Requirements	5473

A.16.7.3	SS-SINR	5473

A.16.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5473

A.16.7.3.1.1	Test Purpose and Environment	5473

A.16.7.3.1.2	Test parameters	5474

A.16.7.3.1.3	Test Requirements	5478

A.16.7.3.2	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5478

A.16.7.3.2.1	Test Purpose and Environment	5478

A.16.7.3.2.2	Test parameters	5479

A.16.7.3.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5483

A.16.7.3.3.1	Test Purpose and Environment	5483

A.16.7.3.3.2	Test parameters	5483

A.16.7.3.3.3	Test Requirements	5488

A.16.7.3.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5489

A.16.7.3.4.1	Test Purpose and Environment	5489

A.16.7.3.4.2	Test parameters	5489

A.16.7.3.4.3	Test Requirements	5493

A.16.7.4	L1-RSRP measurement for beam reporting	5494

A.16.7.4.1	SSB based L1-RSRP measurement for 1 Rx UE	5494

A.16.7.4.2	SSB based L1-RSRP measurement for 2 Rx UE	5494

A.16.7.4.3	CSI-RS based L1-RSRP measurement on resource set with repetition off for 1 Rx UE	5494

A.16.7.4.4	CSI-RS based L1-RSRP measurement on resource set with repetition off for 2 Rx UE	5494

A.16.7.5	E-UTRAN RSRP	5494

A.16.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5494

A.16.7.5.1.1	Test Purpose and Environment	5494

A.16.7.5.1.2	Test parameters	5494

A.16.7.5.1.3	Test Requirements	5500

A.16.7.5.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5501

A.16.7.5.2.1	Test Purpose and Environment	5501

A.16.7.5.2.2	Test parameters	5501

A.16.7.5.2.3	Test Requirements	5507

A.16.7.6	E-UTRAN RSRQ	5508

A.16.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5508

A.16.7.6.1.1	Test Purpose and Environment	5508

A.16.7.6.1.2	Test parameters	5508

A.16.7.6.1.3	Test Requirements	5514

A.16.7.6.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5515

A.16.7.6.2.1	Test Purpose and Environment	5515

A.16.7.6.2.2	Test parameters	5515

A.16.7.6.2.3	Test Requirements	5521

A.16.7.7	E-UTRAN RS-SINR	5522

A.16.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5522

A.16.7.7.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5522

A.17	NR standalone tests with one or more NR cells in FR2 for RedCap	5516

A.17.1	SA: RRC_IDLE state mobility for RedCap	5516

A.17.1.1	Cell re-selection to NR	5516

A.17.1.1.1	Cell reselection to FR2 intra-frequency NR case for 2 Rx	5516

A.17.1.1.1.1	Test Purpose and Environment	5516

A.17.1.1.1.2	Test Parameters	5516

A.17.1.1.1.3	Test Requirements	5520

A.17.1.1.2	Cell reselection to FR2 inter-frequency NR case	5520

A.17.1.1.2.1	Test Purpose and Environment	5520

A.17.1.1.2.2	Test Parameters	5520

A.17.1.1.2.3	Test Requirements	5524

A.17.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5524

A.17.1.1.3.1	Test Purpose and Environment	5524

A.17.1.1.3.2	Test Parameters	5524

A.17.1.1.3.3	Test Requirements	5527

A.17.1.1.4	Cell reselection to FR2 inter-frequency NR case for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE	5527

A.17.1.1.4.1	Test Purpose and Environment	5527

A.17.1.1.4.2	Test Parameters	5527

A.17.1.1.4.3	Test Requirements	5530

A.17.2	SA: RRC_INACTIVE state mobility for RedCap	5530

A.17.2.1	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	5530

A.17.2.1.1	TA validation for CG-SDT in FR2 for RedCap	5530

A.17.2.1.1.1	Test Purpose and Environment	5530

A.17.2.1.1.2	Test Requirements	5534

A.17.3	RRC_CONNECTED state mobility for RedCap	5534

A.17.3.1	Handover for RedCap	5534

A.17.3.1.1	Intra-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	5534

A.17.3.1.1.1	Test Purpose and Environment	5534

A.17.3.1.1.2	Test Parameters	5534

A.17.3.1.1.3	Test Requirements	5536

A.17.3.1.2	Inter-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	5536

A.17.3.1.2.1	Test Purpose and Environment	5536

A.17.3.1.2.2	Test Parameters	5536

A.17.3.1.2.3 Test Requirements	5538

A.17.3.2	RRC Connection Mobility Control for RedCap	5539

A.17.3.2.1	SA: RRC Re-establishment	5539

A.17.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	5539

A.17.3.2.1.1.1	Test Purpose and Environment	5539

A.17.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	5542

A.17.3.2.1.2.1	Test Purpose and Environment	5542

A.17.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	5545

A.17.3.2.1.3.1	Test Purpose and Environment	5545

A.17.3.2.1.3.2	Test Requirements	5547

A.17.3.2.2	Random Access	5548

A.17.3.2.2.1	4-step RA type contention based random access test in FR2 for NR Standalone	5548

A.17.3.2.2.1.1	Test Purpose and Environment	5548

A.17.3.2.2.1.2	Test Requirements	5550

A.17.3.2.2.2	4-step RA type non-contention based random access test in FR2 for NR Standalone	5552

A.17.3.2.2.2.1	Test Purpose and Environment	5552

A.17.3.2.2.2.2	Test Requirements	5554

A.17.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	5556

A.17.3.2.2.3.1	Test Purpose and Environment	5556

A.17.3.2.2.3.2	Test Requirements	5558

A.17.3.2.2.4	2-step RA type non-contention based random access test in FR2 for NR Standalone	5559

A.17.3.2.2.4.1	Test Purpose and Environment	5559

A.17.3.2.2.4.2	Test Requirements	5561

A.17.3.2.3	SA: RRC Connection Release with Redirection	5562

A.17.3.2.3.1	Redirection from NR in FR2 to NR in FR2	5562

A.17.3.2.3.1.1	Test Purpose and Environment	5562

A.17.3.2.3.1.2	Test Parameters	5562

A.17.3.2.3.1.3	Test Requirements	5566

A.17.4	Timing	5566

A.17.4.1	UE transmit timing	5566

A.17.4.1.1	NR UE Transmit Timing Test for FR2	5566

A.17.4.1.1.1	Test Purpose and environment	5566

A.17.4.1.1.2	Test requirements	5569

A.17.4.2	UE timer accuracy	5570

A.17.4.3	Timing advance	5570

A.17.4.3.1	SA FR2 timing advance adjustment accuracy	5570

A.17.4.3.1.1	Test Purpose and Environment	5570

A.17.4.3.1.2	Test Parameters	5570

A.17.4.3.1.3 Test Requirements	5573

A.17.4.2	UE timer accuracy	5573

A.17.4.3	Timing advance	5573

A.17.4.3.1	SA FR2 timing advance adjustment accuracy	5573

A.17.5	Signaling characteristics for RedCap	5573

A.17.5.1	Radio link Monitoring for RedCap	5573

A.17.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	5573

A.17.5.1.1.1	Test Purpose and Environment	5573

A.17.5.1.1.2	Test Requirements	5576

A.17.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	5577

A.17.5.1.2.1	Test Purpose and Environment	5577

A.17.5.1.2.2	Test Requirements	5581

A.17.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	5582

A.17.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	5582

A.17.5.1.4.1	Test Purpose and Environment	5582

A.17.5.1.4.2	Test Requirements	5586

A.17.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	5586

A.17.5.1.5.1	Test Purpose and Environment	5586

A.17.5.1.5.2	Test Requirements	5591

A.17.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	5591

A.17.5.1.6.1	Test Purpose and Environment	5591

A.17.5.1.6.2	Test Requirements	5595

A.17.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	5595

A.17.5.1.7.1	Test Purpose and Environment	5595

A.17.5.1.7.2	Test Requirements	5599

A.17.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	5599

A.17.5.1.8.1	Test Purpose and Environment	5599

A.17.5.1.8.2	Test Requirements	5604

A.17.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	5604

A.17.5.1.9.1	Test Purpose and Environment	5604

A.17.5.1.9.2	Test Requirements	5607

A.17.5.2	Beam Failure Detection and Link recovery procedures	5607

A.17.5.2.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	5607

A.17.5.2.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	5607

A.17.5.2.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	5607

A.17.5.2.3.1	Test Purpose and Environment	5607

A.17.5.2.3.2	Test Requirements	5611

A.17.5.2.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	5611

A.17.5.2.4.1	Test Purpose and Environment	5611

A.17.5.2.4.2	Test Requirements	5615

A.17.5.2.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	5615

A.17.5.2.5.1	Test Purpose and Environment	5615

A.17.5.2.5.2	Test Requirements	5619

A.17.5.3	Active BWP switch for RedCap	5619

A.17.5.3.1	DCI-based and Timer-based Active BWP Switch	5619

A.17.5.3.1.1	NR FR2 DL active BWP switch with non-DRX in SA	5619

A.17.5.3.1.1.1	Test Purpose and Environment	5619

A.17.5.3.1.1.2	Test Requirements	5622

A.17.5.3.2	RRC-based Active BWP Switch	5622

A.17.5.3.2.1	NR FR2 DL active BWP switch of PCell with non-DRX in SA	5622

A.17.5.3.2.1.1	Test Purpose and Environment	5622

A.17.5.3.2.1.2	Test Requirements	5625

A.17.5.4	Active TCI state switch delay	5626

A.17.5.4.1	MAC-CE based active TCI state switch	5626

A.17.5.4.1.1	NR PCell FR2 active TCI state switch for a known TCI state	5626

A.17.5.4.1.1.1	Test Purpose and Environment	5626

A.17.5.4.1.1.2	Test Requirements	5629

A.17.5.4.2	RRC based active TCI state switch	5629

A.17.5.4.2.1	NR PCell FR2 active TCI state switch for a known TCI state	5629

A.17.5.4.2.1.1	Test Purpose and Environment	5629

A.17.5.4.2.1.2	Test Requirements	5633

A.17.5.5	Uplink spatial relation switch delay	5633

A.17.5.5.1	MAC-CE based Spatial Relation switch	5633

A.17.5.5.1.1	 NR PCell FR2 spatial relation associated with known DL-RS	5633

A.17.5.5.1.1.1	Test Purpose and Environment	5633

A.17.5.5.1.1.2	Test Requirements	5636

A.17.5.5.2	RRC based spatial relation switch	5636

A.17.5.5.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	5636

A.17.5.5.2.1.2	Test Requirements	5639

A.17.5.6	UE specific CBW change	5639

A.17.5.6.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	5639

A.17.5.6.1.1	Test Purpose and Environment	5639

A.17.5.6.1.2	Test Requirements	5642

A.17.6	Measurement procedure for RedCap	5643

A.17.6.1	Intra-frequency Measurements	5643

A.17.6.1.1	SA event triggered reporting test without gap under non-DRX	5643

A.17.6.1.1.1	Test purpose and Environment	5643

A.17.6.1.1.2	Test Requirements	5646

A.17.6.1.2	SA event triggered reporting test without gap under DRX	5646

A.17.6.1.2.1	Test purpose and Environment	5646

A.7.6.1.2.2	Test Requirements	5646

A.17.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	5647

A.17.6.1.3.1	Test purpose and Environment	5647

A.17.6.1.3.2	Test Requirements	5651

A.17.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	5651

A.17.6.1.4.1	Test purpose and Environment	5651

A.17.6.1.4.2	Test Requirements	5654

A.17.6.2	Inter-frequency Measurements	5655

A.17.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)	5655

A.17.6.2.1.1	Test Purpose and Environment	5655

A.17.6.2.1.2	Test Requirements	5658

A.17.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)	5658

A.17.6.2.2.1	Test Purpose and Environment	5658

A.17.6.2.2.2	Test Requirements	5662

A.17.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)	5662

A.17.6.2.3.1	Test Purpose and Environment	5662

A.17.6.2.3.2	Test Requirements	5666

A.17.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2) for 2 RX UE	5666

A.17.6.2.4.1	Test Purpose and Environment	5666

A.17.6.2.4.2	Test Requirements	5670

A.17.6.3	L1-RSRP measurement for beam reporting	5670

A.17.6.3.1	SSB based L1-RSRP measurement when DRX is not used	5670

A.17.6.3.1.1	Test Purpose and Environment	5670

A.17.6.3.1.2	Test parameters	5671

A.17.6.3.1.3	Test Requirements	5671

A.17.6.3.2	SSB based L1-RSRP measurement when DRX is used	5671

A.17.6.3.2.1	Test Purpose and Environment	5671

A.17.6.3.2.2	Test parameters	5671

A.17.6.3.2.3	Test Requirements	5673

A.17.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	5673

A.17.6.3.3.1	Test Purpose and Environment	5673

A.17.6.3.3.2	Test parameters	5674

A.17.6.3.3.3	Test Requirements	5676

A.17.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	5677

A.17.6.3.4.1	Test Purpose and Environment	5677

A.17.6.3.4.2	Test parameters	5677

A.7.6.3.3.3	Test Requirements	5679

A.17.6.4.1	SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2) for 2 RX UE	5680

A.17.6.4.1.1	Test Purpose and Environment	5680

A.17.6.4.1.2	Test Requirements	5683

A.17.7	Measurement Performance requirements	5683

A.17.7.1	SS-RSRP	5683

A.17.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	5683

A.17.7.1.1.1	Test Purpose and Environment	5683

A.17.7.1.1.2	Test parameters	5683

A.17.7.1.1.3	Test Requirements	5687

A.17.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	5688

A.17.7.1.2.1	Test Purpose and Environment	5688

A.17.7.1.2.2	Test parameters	5688

A.17.7.1.2.3	Test Requirements	5692

A.17.7.2	SS-RSRQ	5693

A.17.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	5693

A.17.7.2.1.1	Test Purpose and Environment	5693

A.17.7.2.1.2	Test Parameters	5693

A.17.7.2.1.3	Test Requirements	5695

A.17.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell for 2 Rx UE	5695

A.17.7.2.2.1	Test Purpose and Environment	5695

A.17.7.2.2.2	Test parameters	5695

A.17.7.2.2.3	Test Requirements	5697

A.17.7.2.3	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	5697

A.17.7.3	L1-RSRP measurement for beam reporting	5697

A.17.7.3.1	SSB based L1-RSRP measurement	5697

A.17.7.3.1.1	Test Purpose and Environment	5697

A.17.7.3.1.2	Test parameters	5698

A.17.7.3.1.3	Test Requirements	5698

A.17.7.3.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	5698

A.17.7.3.2.1	Test Purpose and Environment	5698

A.17.7.3.2.2	Test parameters	5698

A.17.7.3.2.3	Test Requirements	5699

A.17.7.4	SS-SINR	5699

A.17.7.4	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell for 2Rx UE	5699

A.17.7.4.1.1	Test Purpose and Environment	5699

A.17.7.4.1.2	Test parameters	5699

A.17.7.4.1.3	Test Requirements	5701

A.18	E-UTRA standalone tests for NR RRM for RedCap	5701

A.18.1	RRC_IDLE state mobility	5701

A.18.1.1	Inter-RAT NR Cell re-selection	5701

A.18.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	5701

A.18.1.1.1.1	Test Purpose and Environment	5701

A.18.2.1.1.2	Test Requirements	5706

A.18.2	RRC_CONNECTED state mobility	5706

A.18.2.1	Handover	5706

A.18.2.1.1	E-UTRAN - NR handover in FR1	5706

A.18.2.1.1.1	Test Purpose and Environment	5706

A.18.2.1.1.2	Test Requirements	5711

A.18.2.2	RRC connection release with redirection	5711

A.18.2.2.1	Redirection from E-UTRA to NR FR1 for redcap UE	5711

A.18.2.2.1.1	Test Purpose and Environment	5711

A.18.2.2.1.2	Test Parameters	5711

A.18.2.2.1.3	Test Requirements	5718

A.18.3	Measurement procedure	5718

A.18.3.1	E-UTRA – NR Inter-RAT Measurements	5718

A.18.3.1.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	5718

A.18.3.1.1.1	Test Purpose and Environment	5718

A.18.3.1.1.2	Test Requirements	5724

A.18.3.1.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	5724

A.18.3.1.2.1	Test Purpose and Environment	5724

A.18.3.1.2.2	Test Requirements	5730

A.18.3.1.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	5730

A.18.3.1.3.1	Test Purpose and Environment	5730

A.18.3.1.3.2	Test Requirements	5736

A.18.3.1.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	5736

A.18.3.1.4.1	Test Purpose and Environment	5736

A.18.3.1.4.2	Test Requirements	5742

A.18.3.1.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	5742

A.18.3.1.5.1	Test Purpose and Environment	5742

A.18.3.1.5.2	Test Requirements	5744

A.18.3.1.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	5745

A.18.3.1.6.1	Test Purpose and Environment	5745

A.18.3.1.6.2	Test Requirements	5747

A.18.3.1.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	5748

A.18.3.1.7.1	Test Purpose and Environment	5748

A.18.3.1.7.2	Test Requirements	5750

A.18.3.1.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	5751

A.18.3.1.8.1	Test Purpose and Environment	5751

A.18.3.1.8.2	Test Requirements	5753

Annex B (normative): Conditions for RRM requirements applicability for operating bands	5149

B.1	Conditions for NR RRC_IDLE state mobility	5149

B.1.1	Introduction	5149

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	5149

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	5150

B.1.4	Conditions for measurements on NR intra-frequency cells for cell re-selection for RedCap	5150

B.1.5	Conditions for measurements on NR inter-frequency cells for cell re-selection for RedCap	5152

B.1.6	Conditions for measurements on NR intra-frequency cells for cell re-selection for satellite access	5152

B.1.7	Conditions for measurements on NR inter-frequency cells for cell re-selection for satellite access	5153

B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state	5153

B.2.1	Introduction	5153

B.2.1.1	General	5153

B.2.1.2	Derivation of Minimum SSB_RP values for FR1	5153

B.2.1.3	Derivation of Minimum SSB_RP values for FR2	5153

B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival	5153

B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage	5154

B.2.1.4	Gain to SS-RSRP and CSI-RSRP measurement point for FR1	5155

B.2.1.5	Gain to SS-RSRP and CSI-RSRP measurement point for FR2	5155

B.2.1.5.1	Gain to SS-RSRP and CSI-RSRP measurement point for Rx Beam Peak angle of arrival	5155

B.2.1.5.2	Gain to SS-RSRP measurement point for different frequency	5156

B.2.1.5.3	Alignment of Rough beam to Rx beam Peak	5156

B.2.1.6	Gain to PRS-RSRP measurement point for FR2	5156

B.2.1.6.1	Gain to PRS-RSRP measurement point for Rx Beam Peak angle of arrival	5156

B.2.2	Conditions for NR intra-frequency measurements	5157

B.2.3	Conditions for NR inter-frequency measurements	5158

B.2.4	Conditions for NR L1-RSRP reporting	5159

B.2.4.1	Conditions for SSB based L1-RSRP reporting	5159

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	5160

B.2.5	Conditions for RRC connection release with redirection to NR	5162

B.2.6	Void	5163

B.2.6.1	Void	5163

B.2.6.2	Void	5163

B.2.7	Conditions for SRS-RSRP measurements	5163

B.2.8	Conditions for NR L1-SINR reporting	5164

B.2.8.1	Conditions for L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	5164

B.2.8.2	Conditions for L1-SINR reporting with SSB based CMR and dedicated IMR configured	5165

B.2.8.2.1	L1-SINR reporting with SSB based CMR and dedicated ZP-IMR configured	5165

B.2.8.2.2	L1-SINR reporting with SSB based CMR and dedicated NZP-IMR configured	5165

B.2.8.3	Conditions for L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	5167

B.2.8.3.1	L1-SINR reporting with CSI-RS based CMR and dedicated ZP-IMR configured	5167

B.2.8.3.2	L1-SINR reporting with CSI-RS based CMR and dedicated NZP-IMR configured	5168

B.2.9	Conditions for NR intra-frequency measurements under CCA	5169

B.2.10	Conditions for NR inter-frequency measurements under CCA	5169

B.2.11	Conditions for NR L1-RSRP reporting under CCA	5169

B.2.11.1	Conditions for SSB based L1-RSRP reporting	5169

B.2.12	Conditions for NR CSI-RS based intra-frequency measurements	5170

B.2.13	Conditions for NR CSI-RS based inter-frequency measurements	5171

B.2.14	Conditions for NR PRS-based measurements	5173

B.2.15	Conditions for NR intra-frequency measurements	5174

B.2.16	Conditions for NR inter-frequency measurements	5176

B.2.17	Conditions for NR intra-frequency measurements for satellite access	5178

B.2.18	Conditions for NR inter-frequency measurements for satellite access	5179

B.2.19	Conditions for NR L1-RSRP reporting for satellite access	5179

B.2.19.1	Conditions for SSB based L1-RSRP reporting for satellite access	5179

B.2.19.2	Conditions for CSI-RS based L1-RSRP reporting for satellite access	5179

B.2.20	Conditions for RRC connection release with redirection to NR for satellite access	5180

B.3	RRM Requirements Exceptions	5180

B.3.1	Introduction	5180

B.3.2	Receiver sensitivity relaxation for CA	5180

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	5180

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	5180

B.3.2.2.1	Inter-band carrier aggregation	5180

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	5180

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	5181

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	5181

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	5181

B.3.2.4.1	Intra-band contiguous carrier aggregation	5181

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	5181

B.3.3	Receiver sensitivity relaxation for DC	5181

B.3.3.1	Receiver sensitivity relaxation for EN-DC	5181

B.3.3.2	Receiver sensitivity relaxation for NE-DC	5181

B.3.4	Receiver sensitivity relaxation for SUL	5181

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	5181

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	5182

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	5182

B.4	Conditions for V2X	5182

B.4.1	Test parameters for GNSS signals	5182

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	5182

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	5183

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	5183

B.5	High level test procedure for SAN RRM tests	5183

Annex C (informative): Change history	5184