| 3GPP TR 38.853 V17.1.0 (2023-03) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Introduction of 900MHz NR band for Europe on Rail Mobile Radio (RMR) (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Regulatory background	7

4.1	EC Decision 2021/1703	7

4.2	ECC Decision(20)02	7

4.3	Technical conditions	7

4.3.1	Base stations using wideband technologies	7

4.3.2	Terminals other than cab-radios using wideband technologies	9

5	Frequency band arrangement	9

6	NR system parameters	9

7	RF requirements	10

7.1	BS specific requirements	10

7.1.1	General	10

7.1.2	Transmitter characteristics	10

7.1.2.1	BS maximum output power	10

7.1.2.2	Unwanted emissions	11

7.1.2.1.1	General	11

7.1.2.1.2	OBUE	11

7.1.2.1.3	Tx spurious emissions	11

7.1.3	Receiver characteristics	11

7.1.3.1	Enhanced BS selectivity for band n8	11

7.1.3.2	Rx blocking	11

7.1.3.2.1	CEPT context	11

7.1.3.2.2	Analysis	12

7.1.3.3	Characteristics	12

7.1.3.3.1	Blocking	12

7.1.3.3.2	Intermodulation	12

7.2	UE specific requirements	13

7.2.1	Transmitter characteristics	13

7.2.2	Receiver characteristics	13

8	Performance requirements	15

9	Deployment aspects	15

10	Conclusion	15

Annex A (informative): Change history	16

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is a technical report deals with the use of the Rail Mobile Radio spectrum in the 900MHz frequency band, which was assigned by the ECC Decision (20)02 [1] for the use by the railways in Europe.The purpose is to gather the relevant background information and studies in order to address all the necessary precautions to make the paired spectrum of 874.6-800/919.4-925MHz usable for 5G NR.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	ECC Decision (20)02: "Harmonised use of the paired frequency bands 874.4-880.0 MHz and 919.4-925.0 MHz and of the unpaired frequency band 1900-1910 MHz for Railway Mobile Radio (RMR) ".

[2]	European Commission implementing Decision (EU) 2021/1730 on the harmonised use of the paired frequency bands 874,4-880,0 MHz and 919,4-925,0 MHz and of the unpaired frequency band 1 900-1 910 MHz for Railway Mobile Radio, 28 September 2021.

[3]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[4]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[5]	ECC Report 318, Compatibility between RMR and MFCN in the 900 MHz range, the 1900-1920 MHz band and the 2290-2300 MHz band.

[6]	ECC Report 313; Technical study for co-existence between RMR in the 900 MHz range and other applications in adjacent bands; approved 21 May 2020.

[7]	R4-210431; LS to 3GPP RAN WG4 on the interferer signal definition for the RMR900 BS Rx blocking requirement.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

BWChannel	BS channel bandwidth

FDL,low	The lowest frequency of the downlink operating band

FDL,high	The highest frequency of the downlink operating band

PREFSENS	Conducted Reference Sensitivity power level

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AAS	Advanced Antenna Systems

ACLR	Adjacent Channel Leakage Ratio

ARFCN	Absolute Radio Frequency Channel Number

BS	Base Station

BW	Bandwidth

CEPT	Conférence Européenne des administrations des Postes et Télécommunications

EC	European Commission

ECC	Electronic Communications Committee

EIRP	Effective Isotropic Radiated Power

FDD	Frequency Division Duplex

GSCN	Global Synchronization Channel Number

HST	High Speed Train

MFCN	Mobile/Fixed Communications Networks

NR	New Radio

REFSENS	Reference Sensitivity

RF	Radio Frequency

RMR	Rail Mobile Radio

SCS	Sub-Carrier Spacing

SS	Synchronization Signal

# 4	Regulatory background

## 4.1	EC Decision 2021/1703

On September 28th 2021, the European Commission Decision 2021/1703 [2] established the harmonised conditions for the availability and efficient use of radio spectrum for the Railway Mobile Radio (RMR) in the bands 874.4-880.0 MHz, and 919.4-925.0 MHz. By January 1st 2022, Member States shall designate and make available on a non-exclusive basis the paired frequency bands 874.4-880.0 MHz and 919.4-925.0 MHz for Railway Mobile Radio, in accordance with the technical conditions listed in subclause 4.3.

## 4.2	ECC Decision(20)02

The ECC Decision (20)02 [1] addresses the designation of the paired frequency bands 874.4-880.0 MHz and 919.4-925.0 MHz to be used for Railway Mobile Radio (RMR) on a CEPT wide basis.

By this Decision, the CEPT administrations shall designate the paired frequency bands 874.4-880.0 MHz and 919.4-925.0 MHz for Railway Mobile Radio (RMR) on a non-exclusive basis, according to the technical conditions listed in subclause 4.3.

This Decision entered into force on November 20th, 2020.

## 4.3	Technical conditions

### 4.3.1	Base stations using wideband technologies

The following technical condition for the use of RMR 900MHz applies:

-	Only non AAS Base Stations are considered.

-	The lower edge of the lowest Resource Block shall be ≥ 919.6 MHz.

-	Table 4.3.1-1 and 4.3.1-2 specify the "specific” in-block requirements, based on ECC Decision 20(02) [1].

Table 4.3.1-1: Specific in-block requirements for 5.6 MHz and 5 MHz channels mandatory for uncoordinated deployment

| RMR channel BW | Maximum EIRP |
| --- | --- |
| 5.6 MHz | 62 dBm/5.6 MHz |
| 5 MHz | 64.5 dBm/5 MHz + (fDL – 922.1)×40/3 dB |
| NOTE 1: fDL is the centre frequency in MHz. NOTE 2: NB-IoT in-band operation mode without power boost is allowed. NB-IoT guard-band operation mode and in-band operation mode with power boost are not allowed. |  |

Table 4.3.1-2: Specific in-block requirements for 1.4 MHz and 200 kHz channels mandatory for uncoordinated deployment

| RMR channel BW | Maximum EIRP |
| --- | --- |
| 1.4 MHz | 56 dBm/1.4 MHz + (fDL – 920.2)×40/3 dB (Note 1) |
| 200 kHz | 70.5 dBm/200 kHz + (fDL – 921)×40/3 dB (Note 3) |
| NOTE 1: fDL is the centre frequency in MHz. NOTE 2: Formula applicable to fDL ≤ 921.7 MHz. No specific e.i.r.p. restriction above. NOTE 3: Applicable to NB-IoT standalone operation mode, which is made of one Resource Block. NOTE 4: Formula applicable to fDL ≤ 921.0 MHz. No specific e.i.r.p. restriction above. |  |

Based on the current NR specification and the RMR900 WID scope, the above requirements for 200 kHz, 1.4 MHz and 5.6 MHz channels are not applicable.

Referring to ECC Decision 20(02) table 2 [1], the “general” in-block requirement as presented on table 4.3.1-3 is not mandatory. This requirement is intended to provide guidance to the national administrations in case an upper bound for the EIRP output power would be desired. Therefore, the general in-block requirement in table 4.3.1-3 is not considered as the baseline for general OBUE requirements for NR BS.

Table 4.3.1-3: General in-block requirement – not applicable

| RMR channel BW | Maximum EIRP |
| --- | --- |
| Any | Min {65 dBm/channel; Maximum e.i.r.p. specific to the channel BW} |
| NOTE: the above requirement may be used by an administration in case an upper bound is desired. |  |

Table 4.3.1-4: Out of band requirements

| MHz from block edge (919.4-925 MHz) | EIRP limit |
| --- | --- |
| 0 ≤ Δf < 0.2 | 32.5 dBm/200 kHz |
| 0.2 ≤ Δf < 1 | 14 dBm/800 kHz |
| 1 ≤ Δf < 10 | 5 dBm/MHz |
| NOTE: On a case-by-case basis, at a national level, higher OOB limits may be applied. |  |

Table 4.3.1-5: Baseline requirement

| Frequency Range | EIRP limit |
| --- | --- |
| 880-915 MHz | -49 dBm/5 MHz |
| NOTE: This requirement prevails over out-of-band requirements. |  |

Table 4.3.1-6: Requirements on wideband RMR BS receiver characteristics

| Parameter | Value |
| --- | --- |
| Level of the wanted signal | RefSens + 3 dB |
| Maximum interfering signal in 870-874.4 MHz (Note 1 | -34 dBm |
| NOTE 1: The antenna connector of the radio module is the reference point. The reference sensitivity (RefSens) is the minimum mean power received at the antenna connector at which a specified minimum performance shall be met. NOTE 2: These requirements cover both blocking and third-order intermodulation. NOTE 3: A bandwidth of 200 kHz for the interfering signal is assumed. |  |

### 4.3.2	Terminals other than cab-radios using wideband technologies

Table 4.3.2-1: Terminals and cab-radio requirements using wideband technologies

| Maximum output power | 23 dBm |
| --- | --- |
| ACLR | 30dB minimum |
| Other | Uplink power control is mandatory and shall be activated. |

# 5	Frequency band arrangement

The new RMR band is within the range of FR1 and is proposed as a FDD band (Table 5-1).

Table 5-1: New NR RMR band in FR1

| Band number | UL | DL | Duplex mode |
| --- | --- | --- | --- |
| n100 | 874.4 MHz – 880 MHz | 919.4 MHz – 925 MHz | FDD |

# 6	NR system parameters

The following system parameters are defined for RMR 900MHz band:

Table 6-1: RMR 900 - Channel bandwidth

| NR Band | SCS (kHz) | BS channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 15 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |
| n100 | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6-2: RMR 900 - applicable NR-ARFCN

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n100 | 100 | 174880 – <20> – 176000 | 183880 – <20> – 185000 |

Table 6-3: RMR 900 - applicable SS raster entries

| NR operating band | SS Block SCS | SS Block pattern (NOTE 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n100 | 15 kHz | Case A | 2303 – <1> – 2307 |

# 7	RF requirements

## 7.1	BS specific requirements

### 7.1.1	General

According to ECC Decision (20)02 [1], the BS RF work for RMR900 is limited to the Wide Area Base Stations, determined by the use along railway lines and their corresponding characteristics defined by 3GPP TS 38.104 [4].

In accordance to ECC Decision 20(02) [1], the related studies es were done for the non-AAS BS architectures, defined in 3GPP TS 38.104 [4]. Therefore the RAN4 requirements derivation was limited to the BS type 1-C requirements, only.

As the EIRP limits were defined in the ECC Decision (20)02 [1], consideration of non-AAS BS architecture required to convert those EIRP limits into the conducted requirements assuming a maximum  gain that can vary depending on the used antenna and related antenna gain as well as the potential losses, e.g., feeder. In the present consideration an antenna gain of 17 dBi and 4 dB losses are assumed in accordance to ECC Report 318 [5] resulting to a maximum gain value of GmaxRMR900 = 13 dBi, encompassing internal losses (feeder, etc.) of 0 dB. Other assumptions may lead to other maximum gain values and conducted requirements.

### 7.1.2	Transmitter characteristics

#### 7.1.2.1	BS maximum output power

Based on ECC Decision (20)02 [1], the BS maximum output power for BS operating in band n100 applicable for uncoordinated deployment, should not exceed the Prated,c,AC derived based on table 4.3.1-1 EIRP limits, with the assumption of maximum gain value of the BS antenna gain GmaxRMR900 as:

(64.5 – GmaxRMR900) dBm / 5 MHz + (fDL-922.1) x 40/3 dB = 51.5 dBm/5MHz + (fDL - 922.1) × 40/3 dB,

where GmaxRMR900 is the maximum gain (antenna gain and losses) for the RMR900 BS consideration.

#### 7.1.2.2	Unwanted emissions

##### 7.1.2.1.1	General

Only Category B emission requirements are applicable to NR operation in n100 band.

##### 7.1.2.1.2	OBUE

Emission mask of the OBUE Cat B Option 2 limits in 3GPP TS 38.104 [4] applicable to Europe was compared to the one derived from ECC Decision (20)02 [1], in order to verify the need for additional RMR-specific OBUE requirement introduction.

The following was observed:

-	Both masks were derived with different measurement bandwidths;

-	Emission limits comparison (after measurement bandwidths scaling) revealed that the existing OBUE Cat B Option 2 mask is more stringent only for part of the f offset range (f < 0.2 MHz).

Based on the above motivation the following OBUE requirement are necessary for band n100 in 3GPP TS 38.104 [4], based on conversion of the EIRP limits from ECC Decision (20)02 [1], as summarized in table 7.1.2.1.2-1. The requirement is derived based on table 4.3.1-4, with the assumption of a maximum gain value of the BS GmaxRMR900:

Table 7.1.2.1.2-1: OBUE requirement derivation for n100

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (NOTE) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.2 MHz | 0.1 MHz f_offset < 0.3 MHz | 19.5 dBm | 200 kHz |
| 0.2 MHz f < 1 MHz | 0.6MHz f_offset < 1.4 MHz | 1 dBm | 800 kHz |
| 1 MHz f 10 MHz | 1.5 MHz f_offset < 10.5 | -8 dBm | 1 MHz |
| NOTE: Assuming 13dBi maximum gain (antenna gain and losses) for GmaxRMR900. |  |  |  |

##### 7.1.2.1.3	Tx spurious emissions

The following Tx spurious emission requirements are considered for band n100 in 3GPP TS 38.104 [x], based on conversion of the EIRP limits from ECC Decision (20)02 [1], as summarized in table 7.1.2.1.3-1:

Table 7.1.2.1.3-1: Additional Tx spurious emissions requirement derivation for n100

| Spurious frequency range | Basic limit | Measurement bandwidth |
| --- | --- | --- |
| 880 MHz – 915 MHz | - 49 - GmaxRMR900 = -62 dBm (NOTE) | 5 MHz |
| NOTE: Assuming 13 dBi maximum gain (antenna gain and losses) for GmaxRMR900. |  |  |

### 7.1.3	Receiver characteristics

#### 7.1.3.1	Enhanced BS selectivity for band n8

RAN4 will not specify any requirements to capture the BS enhanced selectivity (band n8) assumption.

#### 7.1.3.2	Rx blocking

##### 7.1.3.2.1	CEPT context

The requirement for performance against an interferer appears in Table 7 of ECC Decision (20)02 [1]. Tracing its origin within CEPT work, it is the result undertaken under ECC report 313 [6] in section 6.1 (“500 MILLIWATT SRD”) and ECC report 313 [6] further indicates “Full technical characteristics of SRD considered in this Report are given in ANNEX 2”. Table 25 of ECC report 313 [6] also provides characteristics of the SRD interferer. The present description is derived from the LS response received from ETSI TC RT in R4-210431 [7].

##### 7.1.3.2.2	Analysis

Several characteristics of the interferer need to be taken into account when defining the conformance test:

-	Table 7 of ECC Decision (20)02 [1] identifies the requirement as “Maximum interfering signal in 870-874.4 MHz”.

-	The corresponding requirement is -34 dBm for 3 dB desensitization which can be converted to -29.2 dBm for 6 dB desensitization.

-	The interferer is less than 200 kHz which may fit to a GSM carrier or a narrowband interferer.

-	The duty cycle of SRD devices is regulated by EU with ECC report 313 [6] indicating ≤ 10% for fixed Network Access Points and ≤ 2.5% otherwise.

-	The requirement covers both “blocking and IM3”.

3GPP conformance tests do not usually consider duty cycle of the interferer and express the interfering signal according to its mean power. In this particular instance that the aggressor system being regulated at EU level and the ECC Decision (20)02 [1] stating a “maximum” interfering signal level, it is assumed to express for the conformance test the interfering signal under a similar scheme of “interfering signal mean power”.

For a duty cycle of 2.5%, a -29 dBm signal max power has a mean power of -45 dBm (i.e. correction by 16 dB).

For a duty cycle of 10%, a -29 dBm signal max power has a mean power of -39 dBm (i.e. correction by 10 dB).

Since the interfering signal power for 10% duty cycle is higher, -39dBm interfering signal mean power has been considered in the 3GPP requirement.

#### 7.1.3.3	Characteristics

##### 7.1.3.3.1	Blocking

For narrowband blocking, the following interferer characteristics need to be considered for band n100 in 3GPP TS 38.104 [4].

-	Duty cycle of the narrowband interferer:	10%

-	Interfering signal mean power:		-39 dBm

-	Wanted signal mean power (dBm):	PREFSENS  + 6 dB

-	Interfering Signal:						5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB

-	Interfering RB centre frequency:		 874.4 MHz -(350 kHz +m*180), m=0, 1, 2, 3, 4, 9, 14, 19

NOTE:	It is assumed that the 5MHz interfering signal does not overlap with the wanted signal channel bandwidth. Otherwise, the minimum offset is 90 kHz (half a RB).

##### 7.1.3.3.2	Intermodulation

For intermodulation, the following interferer characteristics shall be considered for band n100 in 3GPP TS 38.104 [4].

-	Duty cycle of the narrowband interferer:		10%

-	Interfering signal mean power:				-39 dBm

-	Wanted signal mean power (dBm):			PREFSENS + 6 dB

For the interfering signal, it is proposed to use the definitions in accordance with Table 7.7.2-4 of 3GPP TS 38.104 [3] for a 5 MHz channel.

## 7.2	UE specific requirements

### 7.2.1	Transmitter characteristics

The following 3GPP TS 38.101-1 [3] transmitter characteristic changes are expected due to the introduction of 900MHz RMR band:

Table 7.2.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n100 |  |  |  |  |  |  | 23 | ±2 |

Table 7.2.1-2: Requirements for spurious emissions for UE co-existence

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n100 | E-UTRA Band 1, 3, 8, 20, 22, 28, 31, 32, 33, 34, 38, 40, 43, 50, 51, 52, 65, 67, 68, 69, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 7, 41, 42 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |

### 7.2.2	Receiver characteristics

The following 3GPP TS 38.101-1 [3] receiver characteristic changes are expected due to the introduction of 900MHz RMR band:

Table 7.2.2-1: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n100 | 15 | -100 |  |  |  |  |  |  |  |  |  |

Table 7.2.2-2: Uplink configuration for reference sensitivity

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Duplex Mode |
| n100 | 15 | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |

Table 7.2.2-3: In-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 | Case 3 | Case 4 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 | -15 | -38 |
|  | Finterferer (offset) | MHz | -BWChannel/2 – FIoffset, case 1 and BWChannel/2 + FIoffset, case 1 | ≤ -BWChannel/2 – FIoffset, case 2 and ≥ BWChannel/2 + FIoffset, case 2 |  | -BWChannel/2-11 |
| n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n24, n25, n26, n28, n34, n38, n39, n40, n41, n483, n50, n51, n53, n65, n66, n67, n70, n74, n75, n76, n85, n91, n92, n93, n94, n100 | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  |  |

Table 7.2.2-4: Out of-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| NR band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
| n1, n2, n3, | Pinterferer | dBm | -44 | -30 | -15 |
| n5, n7, n8, n12, n13, n14, n18, n20, n24, n25, n26, n28, n30, n34, n38, n39, n40, n41, n485, n50, n51, n536, n65, n66, n67, n70, n71, n74, n75, n76, n85, n91, n92, n93, n94, n100 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |

Table 7.2.2-5: Narrow Band Blocking

| NR band | Parameter | Unit | Channel Bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n24, n25, n26, n28, n30, n34, n38, n39, n40, n41, n48, n50, n51, n53, n65, n66, n67, n70, n71, n74, n75, n76, n85, n100 | Pw | dBm |  | PREFSENS + channel-bandwidth specific value below |  |  |  |
|  |  |  | 16 | 13 | 14 | 16 | 16 |
|  | Puw (CW) | dBm | -55 |  |  |  |  |
|  | Fuw (offset SCS= 15 kHz) 4 | MHz | BWChannel2+0.2SCS+0.5+0.5SCS |  |  |  | NA |
|  | Fuw (offset SCS= 30 kHz)4 | MHz | NA |  |  |  | BWChannel2+BWGB,ChannelSCS+0.5+0.5SCS |

# 8	Performance requirements

The High Speed Train (HST) BS demodulation requirements are to be transparently applied for the NR operation in the RMR900 operating band.

# 9	Deployment aspects

In general, network deployment aspects are out of scope of the RAN4 work. For the derivation of the BS RF requirements it was assumed that the RMR and MFCN base stations are not co-located, and no coordination is necessary for RMR BS deployment.

Co-location of a MFCN BS and a RMR BS requires coordination among the involved parties. The use of spectrum and its related conditions, e.g., E.I.R.P, in accordance to ECC Decision (20)02 Part B [1], are in the responsibility of national regulation and coordination among involved parties.

# 10	Conclusion

The study dealt with how the RMR900MHz spectrum (874.4 - 880 MHz/ 919.4 - 925 MHz) can be used for rail communication using 5G NR applicable in Europe. The corresponding system parameters, BS RF parameter and UE RF parameter were examined:

-	General:	The focus was on the use of RMR900 under uncoordinated deployment conditions, assuming that RMR BS operates independently of other operators e.g. MFCN. Co-location with other operators and resulting coordination is subject to national regulation and is outside the scope of this work item.

-	System Parameters: The RMR900 spectrum block can be used by 5MHz CBW reusing Rel-15 sync raster design. For channel bandwidths smaller than 5 MHz, related study objectives were covered in a separate Rel-18 work item [RP-220401].

-	BS RF: Based on the ECC Decision (20)02 [1], the EIRP limits were analysed accordingly and the necessary conclusions for an uncoordinated operational approach were derived to define conducted power requirements. These include the BS rated output power using corresponding assumptions, the resulting emission values of the transmitter and the specifications of the receiver. Furthermore, the coexistence with other 3GPP radio technologies has been included.

-	UE RF:	Based on the ECC Decision (20)02 [1], the EIRP limits were analysed accordingly, and the necessary conclusions were derived for the use of Power Class 3 (23dBm).

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-08 | RAN4#100-e |  |  |  |  | Initial baseline | 0.0.1 |
| 2021-11 | RAN4#101-e | R4-2114370 |  |  |  | TP to 38.853 on 900MHz to RMR RAN4 system parameters | 0.1.0 |
| 2022-01 | RAN4# 101-bis-e | R4-2120036 |  |  |  | RMR 900MHz band – TP to TR 38.853 | 0.2.0 |
| 2022-01 | RAN4# 101-bis-e | R4-2120041 |  |  |  | TP to 38.853 on 900MHz RMR RAN4 UE RF requirements | 0.2.0 |
| 2022-01 | RAN4# 101-bis-e | R4-2120680 |  |  |  | TPs to TR 38.853: BS RF related agreements for RMR900 | 0.2.0 |
| 2022-03 | RAN4#102-e | R4-2202246 |  |  |  | TR_38.853_changes_clause 7.1 | 0.3.0 |
| 2022-03 | RAN4#102-e | R4-2200702 |  |  |  | On RMR900 sensitivity | 0.3.0 |
| 2022-03 | RAN4#102-e | R4-2203053 |  |  |  | Proposed changes for clause 9 in 3GPP TR 38.853 | 0.3.0 |
| 2022-03 | RAN4#102-e | R4-2203054 |  |  |  | Cross-check on the implementation of ECC Decision (20)02 for RMR900 | 0.3.0 |
| 2022-03 | RAN4#102-e | R4-2203056 |  |  |  | TP to TR 38.853 for RMR900 | 0.3.0 |
| 2022-04 | RAN4#102-e | R4-2207267 |  |  |  | TP BS RF conducted requirements for n100 | 0.4.0 |
| 2022-05 | RAN4#103-e | R4-2210884 |  |  |  | TP 900MHz RMR band – conclusion | 0.5.0 |
| 2022-09 | RAN#97-e | RP-222515 |  |  |  | TR 38.853 v1.0.0 on Introduction of 900MHz NR band for Europe for Rail Mobile Radio (RMR) | 1.0.0 |
| 2022-09 | RAN#97-e |  |  |  |  | Under change control | 17.0.0 |
| 2023-03 | RAN#99 | RP-230517 | 0001 |  | F | Correction to TR 38.853 | 17.1.0 |