| 3GPP TR 38.884 V2.0.1 (2022-03) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Study on enhanced test methods for FR2 NR UEs; (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	8

2	References	9

3	Definitions of terms, symbols and abbreviations	10

3.1	Terms	10

3.2	Symbols	10

3.3	Abbreviations	10

4	General	12

5	UE RF testing methodology enhancements	13

5.1	High DL power and low UL power	13

5.1.1	General	13

5.1.2	Beam management sensitivity study of NF based solutions	14

5.1.2.1	Simulation assumptions	14

5.1.2.2	Simulation results	15

5.1.3	Manufacturer declarations	17

5.1.4	Permitted Methodologies: CFFNF, CFFDNF and CFFdeltaNF	24

5.1.4.1	Asymptotic Expansion Approach for CFFNF	26

5.1.4.2	Test Procedures for CFFDNF, CFFNF and CFFdeltaNF	28

5.1.4.3	Simulation Assumptions	34

5.1.4.4	Simulation results for CFFDNF	35

5.1.4.5	Simulation results for CFFNF (using Black & White-Box Approach)	52

5.1.4.6	Simulation results for CFFNF (using Black-Box Approach)	54

5.1.4.7	Simulation results for sensitivity of CFFNF to relative measurement uncertainties	58

5.1.4.8	Simulation results for CFFdeltaNF	59

5.1.4.9	Simulation results for Influence of Noise	75

5.1.4.10	Simulation Results for offset error MU	78

5.1.5	Applicability of NF methodologies	80

5.1.6	Improvement of permitted methods	83

5.2	Polarization basis mismatch between the TE and DUT	84

5.2.1	General	84

5.2.2	Enhanced test method for EIRP measurement	85

5.2.2.1	TPMI method	85

5.2.2.2	Applicability of TPMI side condition method	85

5.2.2.3	Alternative test method	86

5.2.3	Enhanced test method for UL demodulation measurement	86

5.2.3.1	Test equipment Zero-forcing MIMO receiver	86

5.2.3.1.1	Method 1	86

5.2.3.1.1.1	Method 1 description	86

5.2.3.1.1.2	Method 1 MIMO Equalization	88

5.2.3.1.1.3	Method 1 Maximum Ratio Combining	88

5.2.3.1.1.4	Method 1 Layer processing	89

5.2.3.1.2	Method 2	89

5.2.3.1.2.1	Method 2 description	89

5.2.3.1.2.2	Method 2 MIMO Equalization	90

5.2.3.1.2.3	Method 2 Maximal Ratio Combining	91

5.2.3.1.2.4	Method 2 EVM equalizer flatness	92

5.2.3.1.2.5	Method 2 channel invertibility considerations	92

5.2.3.1.2.6	Method 2 DC (LO) cancellation	93

5.2.3.1.3 Analysis of Method 1 & 2	94

5.2.3.1.3.1 Comparison of both Methods	94

5.2.3.1.3.2 Conclusion	96

5.3	Inter-band (FR2+FR2) CA	96

5.3.1	General	96

5.3.2	Impact of multiple test antennae	97

5.3.2.1	PSD imbalance with DL signals from test equipment	97

5.3.2.1.1	DL PSD towards UE supporting independent beam management (IBM)	97

5.3.2.1.2	DL PSD towards UE supporting common beam management (CBM)	98

5.3.2.2	Impact of off-focus test system antennae	100

5.3.2.2.1	Quality of quiet zone (QoQZ)	100

5.3.2.2.2	Rx beam profiles with an independent beam management (IBM) UE	106

5.3.2.2.3	Propensity to trigger incorrect beam in CBM UE	107

5.3.2.2.4	Spherical coverage measurement simulation with common beam management (CBM) UE	108

5.3.2.3	Summary on applicability of offset antenna test system	110

5.3.2.4	Points to design the FR2 OTA test system with offset test antennae	110

5.3.3 	Inter-band testing ramifications	111

5.3.3.1 	Single antenna	112

5.3.3.2 	Multiple antennae	112

5.4	Extreme temperature conditions	112

5.4.1	ETC test system	112

5.4.2	Calibration procedure	113

5.4.3	Test procedure	113

5.4.4	Temperature tolerance limit of ETC test system	113

5.5	Extension of frequency applicability for band n262	114

5.6	Extension of frequency applicability for FR2-2	114

5.6.1	Permitted test methods	114

5.6.2	Enhanced test methods	114

5.6.2.1	High DL power and low UL power	114

5.6.2.2	Polarization basis mismatch between the TE and DUT	114

5.6.2.3	Inter-band (FR2+FR2) CA	114

5.6.2.4	Extreme temperature conditions	114

5.6.2.5	Test time reduction	114

6	UE RRM testing methodology enhancements	115

6.1	Extension of frequency applicability for band n262	115

6.2	Extension of frequency applicability for FR2-2	115

6.2.1	General	115

6.2.2	Noc methodology	115

6.2.3	Maximum SNR derivation	115

7	UE demodulation testing methodology enhancements	117

7.1	Extension of frequency applicability for band n262	117

7.2	Extension of frequency applicability for FR2-2	119

7.2.1	General	119

7.2.2	Noc methodology	119

7.2.3	Maximum SNR derivation	119

8	Test time reduction	122

8.1	General	122

8.2	New measurement grid	122

8.2.1	New measurement grids based on 4x2 antenna pattern assumption	122

8.2.1.1	Beam Peak Search Measurement Grid	123

8.2.1.2	Spherical Coverage Measurement Grid	125

8.2.1.3	TRP Measurement Grid	126

8.2.2	Applicability of the 4x2 measurement grids	129

8.3	RSRP(B) based RX beam peak search	130

8.3.1	Test procedure	130

8.3.2	RSRP(B) accuracy	130

8.4	Single link polarization measurement	130

8.4.1	Test procedure	130

8.4.2	Applicability of Single link polarization measurement	131

8.5	Other methods	132

8.5.1	Fast Spherical Coverage Method	132

8.5.1.1	General	132

8.5.1.2	Tx Fast Spherical Coverage Method	132

8.5.1.3	Rx Fast Spherical Coverage Method	133

8.5.2	Non-Uniform TRP Measurement Grids	134

9	Propagation conditions	136

9.1	Extension of frequency applicability for FR2-2	136

Annex A: Environment conditions	138

A.1	Operating voltage	138

A.2	Temperature	138

Annex B: Measurement uncertainty	139

B.1	Measurement uncertainty budget for UE RF testing methodology	139

B.1.1	High DL power and low UL power	139

B.1.1.1	Uncertainty Contributions	139

B.1.1.2	Uncertainty Contributions descriptions	142

B.1.1.2.1	DUT antenna location estimation	142

B.1.1.2.2	Probe antenna pattern	142

B.1.1.2.3	EIRP measurement error in NF	143

B.1.1.2.4	TRP measurement error in NF	143

B.1.1.2.5	Near-field interaction between probe antenna and DUT antenna	143

B.1.1.2.6	Influence of power measurement uncertainty	143

B.1.1.2.7	Influence of noise	143

B.1.1.3	Uncertainty assessment	143

B.1.2	Polarization basis mismatch between the TE and DUT	144

B.1.3	Inter-band (FR2+FR2) CA	144

B.1.4	Test system for ETC	144

B.1.5	Measurement uncertainty budget for FR2-2 test methods	145

Annex C (informative): Change history	146

For definitive guidance on drafting 3GPP TSs and TRs, see 3GPP TS 21.801 supplemented by the 3GPP web page http://www.3gpp.org/specifications-groups/delegates-corner/writing-a-new-spec.

Ensure all blue guidance text is removed before submitting the TS/TR to the TSG for approval.

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In drafting the TS/TR, pay particular attention to the use of modal auxiliary verbs! TRs shall not contain any normative provisions.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The objectives of this study are to enhance the FR2 RF testing methodology and to quantify the impact of the enhancements on the UE performance, as related to the polarization basis mismatch between the test equipment and UE and to add support for testing under extreme temperature conditions.

The development of testing methodology enhancements proceeds within the following scope:

-	In general

-	Target the testing and calibration aspects of the permitted methods for FR2 UE RF testing and the preliminary assessment of measurement uncertainty (Clause 5.2 and Annex B of TR38.810)

- 	The test methodologies and procedures shall be applicable for different device types and power classes with DUT size defined in the TR 38.810.  Prioritize the study to PC3 for aspects related to DUT size, and limit the study to free space conditions

-	The study item outcomes shall capture the efficacy of the enhancements

-	Objectives related to regulatory test cases shall be prioritized

The detailed objectives are:

1.	Define test methodology for high DL power and low UL power test cases

-	Considering path loss reduction, measurement antenna gain improvement, DUT positioning improvement, and MU improvement

-	Considering NFTF (defined in Clause 5.2 of TR38.810) and direct near field test methodologies as possible alternative methods

-	Other approaches are not precluded

-	Study preliminary assessment of measurement uncertainty of new alternative methods

2.	Define solutions to minimize the impact of polarization basis mismatch between the TE and DUT on the RF testing

-	Considering polarization basis mismatch between the test equipment and UE and UE implementations which may be impacted by this mismatch

-	Study EIS test metric which can apply to different UE RF implementations considering downlink polarization sweep enhancement

- 	Limit the study of this objective to the permitted UE RF methods defined in Clause 5.2 of TR38.810

-	Possible enhancements may be described as

-	Downlink polarization sweeping by the test equipment (i.e. introducing an additional degree of freedom for polarization alignment of the measurement antenna)

-	The use of circular polarization to perform measurements

-	Coherent combining and demodulation of orthogonally polarized received signals in the test equipment

-	Uplink polarization sweeping by the test equipment to search for the optimal polarization angle to receive and demodulate the signal transmitted by the UE

-	Considering NFTF (defined in Clause 5.2 of TR38.810) test methodology for EIS measurement

-	TPMI side condition method, where TPMI side conditions are applicable to Rel-16 (and higher) UEs

-	Test mode to trigger TX diversity

-	Other approaches are not precluded

3.	Study testability enhancements to support the verification of RF requirements for inter-band (FR2+FR2) CA

-	Work on inter-band DL CA is prioritized

-	Whether the test setup shall be restricted to emulating the signal from the same direction for the aggregated bands shall be aligned with the UE RF architecture assumption taken in the work item on NR RF Requirement Enhancements for FR2 [UID 830189]

4.	Support extreme temperature conditions for all applicable FR2 UE RF test cases

-	Considering beam peak search, spherical coverage, and total radiated power procedures

- 	Limit the study of this objective to the permitted UE RF methods defined in Clause 5.2 of TR38.810

-	Study preliminary impacts on system measurement uncertainty under extreme temperature conditions

5.	Study testability enhancements to reduce test time

-	Including RF test method enhancement with reduced test time, and possible test time saving approach for UE Demodulation test and RRM test

6.	Study testability aspects for the introduction of the new band n262

-	Considering the extension of frequency applicability of the permitted methods in TR38.810 from 43.5 GHz up to at least 48.2 GHz

-	Considering the extension of frequency applicability of the test methodology enhancements in Objectives 1 through 5 above

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-2: "User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[3]	3GPP TR 38.810: "NR; Study on test methods".

[4]	3GPP TS 38.211: "NR; Physical channels and modulation".

[5]	3GPP TS 38.212: "NR; Multiplexing and channel coding".

[6]	3GPP TS 38.521-2: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone".

[7]	3GPP TR 38.903: " Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases".

[8]	L. Anttila, A. Brihuega, M. Valkama. On Antenna Array Out-of-Band Emissions, IEEE Wireless Communications Letters, Dec 2019

[9]	H. Li, X. Li, C. Wei, The analysis of the performance of multi-beamforming in memory nonlinear power amplifier, EURASIP Journal on Advances in Signal Processing, 2014

[10]	3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AoA	Angle of Arrival

BP	Beam Peak

CA	Carrier Aggregation

CBW	Channel Bandwidth

CFFNF	Combined Far-Field/Near-Field

CFFDNF	Combined Far-Field/Direct-Near-Field

DFF	Direct Far Field

DNF	Direct Near Field

DUT	Device Under Test

EIS	Effective Isotropic Sensitivity

EIRP	Effective (or equivalent) isotropic radiated power

EPRE	Energy per resource element

ETC	Extreme Temperature Condition

EVM	Error Vector Magnitude

FF	Far-Field

FR1	Frequency Range 1

FR2	Frequency Range 2

FS	Free Space

IBM	Independent Beam Management

IFF	Indirect Far Field

MU	Measurement Uncertainty

NF	Near-Field

NFTF	Near Field to Far-field

NSA	Non-standalone

OTA	Over The Air

QZ		Quiet Zone

RSRP	Reference signal receive power

SNR	Signal-to-Noise Ratio

SS	System Simulator

SS-RSRPB	SS reference signal received power per branch

TRP	Total Radiated Power

UBF	UE beam lock function

UE	User Equipment

# 4	General

Editor’s note: general aspects related to the scope of the study or common study outcomes can be captured in this clause.

The enhanced test methods defined in the following clauses are not only applicable to current release, the general applicability can be categorized as following:

1.	If a test case or requirement was not-testable in an older Release (e.g. Rel-15), and a new test method or procedure unblocks it in Rel-17, then the method can be said to be release-independent and would apply to the older release.

2.	If we call one of the enhanced test methodologies as a Rel-17 test equipment feature, then a test equipment setup implementing the earlier release conformance test specification should be allowed to bring in a Rel-17 feature, provided that it meets all of the applicable Rel-17 requirements for that feature.

3.	When there is a core requirement change (or modification/relaxation) in, e.g. Rel-17, to enable testing/unblock testability issues, then the requirement change should be applicable only from Rel-17 onwards.

4.	When the enhanced test methodology applies to a UE feature supported only from a specific release, the test method becomes applicable only from that release onwards.

# 5	UE RF testing methodology enhancements

Editor’s note: testing and calibration aspects of the permitted methods for FR2 UE RF testing and the preliminary assessment of measurement uncertainty (Clause 5.2 and Annex B of TR38.810) define the baseline UE RF methodology for the purpose of this study.

## 5.1	High DL power and low UL power

### 5.1.1	General

The investigation of high DL power and low UL power enhancements to the FR2 test methodology includes the following aspects:  scope of test cases with high DL power and low UL power issues, enhanced test systems, including the investigation of non-permitted systems, enhancements to permitted methods, manufacturer declarations, beam management sensitivity of the DUT in near-field test system environments, and path loss comparison across system types.

Table 5.1.1-1 below provides a summary of the test cases and testability issues.

Table 5.1.1-1: Summary of test cases and testability issues

| Clause | Requirement | Testability issue | Test Metric |
| --- | --- | --- | --- |
| 6.3.1 | Minimum output power | Low UL power | EIRP (Link=TX beam peak direction, Meas=Link angle). |
| 6.3.2 | Transmit OFF power | Low UL power | TRP (Link=TX beam peak direction, Meas=TRP grid) |
| 6.5.1 | Occupied bandwidth | Low UL power | OBW (Link=TX beam peak direction, Meas=Link angle) |
| 6.5.2.3 | Adjacent channel leakage ratio | Low UL power | TRP (Link=TX beam peak direction, Meas=TRP grid). |
| 6.5.3.2 | Additional spurious emissions | Low UL power | TRP (Link=TX beam peak direction, Meas=TRP grid). |
| 7.4 | Maximum input power | High DL power | EIS (Link=RX beam peak direction, Meas=Link angle). |
| 7.5 | Adjacent channel selectivity (case 1) | High DL power | EIS (Link=RX beam peak direction, Meas=Link angle) |
| 7.5 | Adjacent channel selectivity (case 2) | High DL power | EIS (Link=RX beam peak direction, Meas=Link angle) |
| 7.6.2 | In-band blocking | High DL power | EIS (Link=RX beam peak direction, Meas=Link angle) |
| 7.9 | Receiver spurious emissions | Low UL power | TRP (Link=TX beam peak direction, Meas=TRP grid). |

The investigation of test methodology enhancements to strive to reduce the testability issues which were identified includes study of the feasibility of enhancing test systems which are permitted in TR38.810 [3] as well as test systems which are not permitted.  Non-permitted test systems according to TR38.810 [3] are not required to verify all requirements in TS38.101-2 [2].  The candidate test systems are limited to near-field (NF) based solutions and include the following solutions:

-	The Direct near-field (DNF) system assumes that all measurements and call setups are performed with a measurement probe in the NF of the DUT.

-	The Combined far-field/near-field (CFFNF) system utilizing a transform-based approach assumes that the UE beamlock function (UBF) activation is performed towards the FF beam peak direction based on the far-field method and then test case procedures are performed with measurement probe(s) in the NF of the DUT.

-	Combined far-field/direct-near-field (CFFDNF) system assumes that the UE beamlock function (UBF) activation is performed towards the FF beam peak direction based on the far-field method and then test case procedures are performed based on the direct near-field method.

-	Combined far-field/delta-near-field (CFFdeltaNF) system utilizing a relative measurement approach assumes that the UE beamlock function (UBF) activation is performed towards the FF beam peak direction based on the far-field method, a reference test case without testability issues is performed both in FF and NF to obtain the relative correction factor, and the final test case procedure are performed in the NF and compensated with the relative correction factor.

The applicability of these NF methodologies is further outlined in Clause 5.1.4.

### 5.1.2	Beam management sensitivity study of NF based solutions

#### 5.1.2.1	Simulation assumptions

For NF based solutions, where beam peak search is necessary to perform all applicable test case procedures, an evaluation of UE beam management sensitivity to magnitude/phase variation of the DL signal is needed.  Two assumptions are made about the NF based system:

-	Beam peak search is performed in the NF (i.e. DNF system); OR

-	Beam peak search is first performed in the FF/IFF and test case is executed in the NF (i.e. CFFNF system).

Using the spherical coverage measurement grid assumptions shown in Table 5.1.2.1-1, evaluations were performed of the UE beam management sensitivity in terms of simulated radiated performance metrics for each of the assumptions.

Table 5.1.2.1-1: Beam management sensitivity simulation assumptions

| Parameter | Value | Notes |
| --- | --- | --- |
| Spherical coverage Measurement Grids baseline assumption | Annex G.1.1 in TR38.810 |  |
| Antenna array | - 8x2 and 4x1 - Antenna element HPBW: {260/130, 90/90} deg | Element near-field assumption is implementation specific |
| Simulated DUT | Two antenna arrays are integrated in the UE for the spherical coverage analyses - Antenna panels are studied with Nz x Ny with Nz>Ny, e.g., 8x2 corresponds to Nz = 8 and Ny = 2 - The implementation loss for the antenna near the front is 0dB less than that for the antenna near the back - The antenna in the back is on the opposite side of the UE (mirrored around (0,0,0)). | See Figure 5.1.2.1-1 for example positions of two antenna arrays |
| Beam steering | - In the xy plane, assume 45º beam steering granularity (AZ from -45º to +45º) - In the xz plane, assume 22.5o beam steering granularity (EL from -90º to 90º) |  |
| Offsets | - Various antenna offsets (yoffset, zoffset) beyond 7.5cm in radius (12.5cm max) - For TRP analysis, model random antenna offsets anywhere within the 30cm spherical QZ | Offset is defined with respect to the center of antenna array |
| Range Lengths | - 30cm, 20m (more range lengths are not precluded) - Goal is to eventually determine min. range length and MU for performing spherical coverage tests in DNF | Defined as distance between centre of QZ/positioning axes and measurement probe |
| Test methodology | - CFFDNF/DNF (while taking path loss offsets into account) - CFFNF |  |
| Sampling grid | Study finer than 7.5deg step size for constant-step size grids | Parametric studies to show convergence for the selected assumption |

Figure 5.1.2.1-1 below illustrates example positions of two antenna arrays in the simulated DUT.

Figure 5.1.2.1-1: Simulated DUT antenna assumptions for beam management sensitivity study

#### 5.1.2.2	Simulation results

Table 5.1.2.2-1 below summarizes the results from simulations of beam management sensitivity of a DNF system (i.e. beam peak search is performed in the NF).

Table 5.1.2.2-1: Beam management sensitivity results of a DNF system

| Company label | Swept parameters | Beam management performance maximum ∆ relative to reference (dB) |  |  | Notes |
| --- | --- | --- | --- | --- | --- |
|  |  | Beam peak | 50% CDF | TRP |  |
| Company A | Array: 8x2 Range: {0.2, 0.4, 0.8} m Offset: {0, 0.05, 0.10} m HPBW: {90/90} | 2.5 | Not analysed | Not analysed | There is approximately 2.5 dB of BP error when range length is reduced to 0.2m in presence of the module offset mentioned above. There is also significant perturbation of the CDF curve. CDF statistics start to converge when the range length is at least 4 times the offset. |
| Company B | Array: 8x2, 4x1 Range: {0.25, 0.3, 0.45, 20} m Offset: {0.125 in y, 0.125 in z, 0.09 in y & z} m HPBW: {260/130} | 7.0 | 1.0 | TRP analysed separately | The EIRP beam peak (100%-ile EIRP) and direction cannot be measured accurately with the direct NF methodology |
| Company B | Array: 8x2 Range: 0.2 m Offset: 0.15 m in x, y, z HPBW: {260/130} |  |  | 0.66 dB systematic 0.46 dB RSS’ed | Large uncertainties can be observed for TRP for measurements performed in the NF utilizing the black back box approach |
| Company C | Array: 4x1 Range: {100, 4.2, 0.9, 0.45, 0.3} m Offsets: not specified Full phone model (including the PCB and phone house) has been considered | 0.3 | 0 | Not clear whether 0.1 or 0.4 | Figure of merits such as EIRP, TRP, and Spherical Coverage are not influenced dramatically from range length |
| Company B | Array: {4x1, 8x2} Range: 0.25 m Offset: {0, 0.125, 0.9} m in y, z HPBW: {90/90} | 4.2 |  |  | UE selected different beam between NF beam peak direction and FF beam peak direction |
| Company B | Array: {4x1, 8x2} Range: 0.25 m Offset: {0, 0.53, 0.75} m in x, y, z HPBW: {90/90} | 10.4 |  |  | UE select the same beam in the NF as in the FF more often, we still see concerning trends with the peak EIRP deltas |
| Company B | Reuse assumptions used by Company A: Array: 8x2 Range: {0.2, 0.4, 0.8} m Offset: {0, 0.05, 0.10} m HPBW: {90/90} | 2.5 | 1.2 |  | Simulations were performed to establish alignment with another company |

Table 5.1.2.2-2 below summarizes the results from simulations of beam management sensitivity of a CFFNF system (i.e. beam peak search is first performed in the FF/IFF and test case is executed in the NF).

Table 5.1.2.2-2: Beam management sensitivity results of a CFFNF system

| Company and reference | Swept parameters | Beam management performance maximum ∆ relative to reference (dB) |  |  | Notes |
| --- | --- | --- | --- | --- | --- |
|  |  | Beam peak | 50% CDF | TRP |  |
| Company B (“Black box with transform approach”) | Array: 8x2, 4x1 Range: {0.22 – 0.30} m Offset: {0, 0.50, 0.10, 0.125} m | Max µ = 0.2 Max σ = 0.3 | Not analysed | Not analysed | These results were obtained using a transform-based approach to correct the incurred path loss. Feedback from industry is requested whether to continue efforts in terms of simulations and empirical investigations on this enhanced NF methodology with transform utilizing black-box approach |
| Company B (“Black & White box with transform approach”) | Array: 8x2, 4x1 Range: {0.22 – 0.30} m Offset: {0, 0.50, 0.10, 0.125} m | Max µ = 0.1 Max σ = 0.3 | Not analysed | TRP analysed separately | These results were obtained using a transform-based approach to correct the incurred path loss. Feedback from industry is requested whether to continue efforts in terms of simulations and empirical investigations on this enhanced NF methodology with transform utilizing the white&black-box approach |
| Company B (“TRP with compensation for antenna offset”) | Array: 8x2 Range: 0.2 m Offset: 0.15 m in x, y, z HPBW: {260/130} |  |  | 0.02 dB systematic 0.21 dB RSS’ed | These results were obtained using the DNF methodology with declared offset; alternatively, these results could be obtained using a transform based approach to estimate the phase centre offset. With the offset of the antenna array known, e.g., estimated with the enhanced NF methodology introduced in this contribution, very accurate TRP measurements in the NF can be made with a TRP offset compensation approach |
| Company C | Array: 4x1 Range: {100, 4.2, 0.9, 0.45, 0.3} m Offsets: not specified Full phone model (including the PCB and phone house) has been considered | 0.3 | 1.0 | 0.8 | These results were obtained using the DNF methodology. Figure of merits such as EIRP, TRP, and Spherical Coverage are not influenced dramatically from range length Full phone model (including the PCB and phone house) has been considered |

### 5.1.3	Manufacturer declarations

If a manufacturer declaration is used to inform or optimize a test system parameter, and the DUT is positioned in the test system according to parameters which are informed by this declaration, then the DUT is measured assuming a “white box” configuration.  If no manufacturer declaration is used, and the DUT is positioned in the test system according to common procedures, then the DUT is measured assuming a “black box” configuration.

Black box testing requires no knowledge which antenna panel is active at any given time and the detailed location of the active panel within the DUT. In this test configuration, the geometric centre of the DUT is aligned with the centre of the quiet zone as illustrated in Figure 5.1.3-1.

Figure 5.1.3-1: Illustration of black box approach

White box testing on the other hand requires the manufacturer declaration of detailed locations of all antenna panels and which antenna panel is active in any UL/DL test direction In this test configuration, the centre of the radiating aperture (of the active panel) is aligned with the centre of the quiet zone as illustrated in Figure 5.1.3-2.

Figure 5.1.3-2: Illustration of white box approach

The black&white-box approach combines the advantages of both the black and white-box approaches where the antenna phase centre offset(s) are declared, i.e., white-box approach, but the geometric centre of the DUT is aligned with the centre of the QZ, i.e., black-box approach.

The following paragraphs provide further information on the need for the various vendor declarations with the help of sample illustrations. Here, a DUT with three antenna panels is considered shown schematically in Figure 5.1.3-3 on the left. The right side shows coverage sectors and the corresponding antenna panels the DUT would select if the DL was presented from within those sectors. In this example, the red antenna panel would yield the TX beam peak in the horizontal direction; this direction would be identified following the TX beam peak search. For simplicity, most of the arguments in the next few paragraphs are applied to testing in the FF but they can be applied to testing in the NF as well.

Figure 5.1.3-3: Illustration of Sample DUT with three antenna panels

The beam peak search or spherical coverage test case of the DUT utilizing the black-box approach, i.e., none of the antenna offsets are known/declared, is illustrated in Figure 5.1.3-4. Here, the geometric centre of the DUT is aligned with the centre of the QZ (yellow circle). The (green) beam peak search grid points sample the EIRP around the DUT.

Figure 5.1.3-4: Illustration of beam peak search of sample DUT utilizing black-box approach.

Test cases without a 3D scan, e.g., EIRP/EIS test case towards the known TX/RX beam peak direction, utilizing the black-box approach are illustrated in Figure 5.1.3-5. Here, the geometric centre of the DUT is aligned with the centre of the QZ (yellow circle) and the TX beam peak direction is known from a previous beam peak search measurement, e.g., from an IFF system. Hence, the single (green) FF grid point is aligned with the FF TX beam peak direction.

Figure 5.1.3-5: Illustration low UL power test case along TX BP direction of sample DUT utilizing black-box approach.

For the white-box measurement approach, the level of information provided in vendor declarations largely depends on the purpose of test case coverage. If the white-box approach is leveraged for all conformance test cases including the beam peak searches, the total number of panels and the phase centre offsets of each panel need to be declared. Additionally, vendors would have to declare which antenna panel is active for each grid point or test sectors so that the respective antenna panel is aligned with the centre of the QZ during testing. This approach is further illustrated in Figure 5.1.3-6. To sample EIRPs on all beam peak search grid points, three different device positions have to be applied, i.e., for the angular range covering the

-	red grid points (declared by OEM), the red antenna panel (location declared by OEM) has to be aligned with the centre of QZ (yellow circle)

-	purple grid points (declared by OEM), the purple antenna panel (location declared by OEM) has to be aligned with the centre of QZ (yellow circle)

-	blue grid points (declared by OEM), the blue antenna panel (location declared by OEM) has to be aligned with the centre of QZ (yellow circle)

Figure 5.1.3-6: Illustration of beam peak search of sample DUT utilizing white-box approach.

In summary, the information that would have to be declared by the OEMs if the white-box approach is utilized for all conformance test cases is tabulated in Table 5.1.3-1.

Table 5.1.3-1: Sample Vendor Declaration for white box approach supporting all conformance test cases

| Number of Antenna Panels in DUT | # |  |
| --- | --- | --- |
| Antenna Panel # | Phase-centre offset from geometric centre of DUT | Range of Angles covered by Antenna Panel |
| 1 | (xoff1, yoff1, zoff1) | (start1 to end1,start1 to end1) |
| 2 | (xoff2, yoff2, zoff2) | (start2 to end2,start2 to end2) |
| … | … | … |
| N | (xoffN, yoffN, zoffN) | (startN to endN,startN to endN) |

Assuming the enhanced test methodology needs to perform beam peak searches and a white box approach was selected, the DUT should be measured in several positions inside the test volume, where two options could be considered:

a.	DUT is placed manually in the corresponding off-center positions. This will likely result in significant test time increase and additional MU due to inaccuracies in the alignment of the DUT.

b.	x-y-z positioning systems are needed to fully automate testing based on the knowledge of which antenna panel is active in any given UL/DL test direction, as outlined in Figure 5.1.3-6. This will in effect likely result in significant signal ripple and near field coupling effects which is expected to degrade the quality of QZ MU which could offset the offset MU a white box approach eliminates. Such positioning system will furthermore increase test system complexity from a SW and HW perspective as well as test time.

Test cases without a 3D scan, e.g., EIRP/EIS test case towards the known TX/RX beam peak direction, utilizing the white-box approach is illustrated in Figure 5.1.3-7. Here, the phase centre of the red panel (yielding beam peak radiation) of the DUT is aligned with the centre of the QZ (yellow circle) and the TX beam peak direction is known from a previous beam peak search measurement; thus the single (green) grid point is aligned with the FF TX beam peak direction. In this case, only the location of the one antenna panel that yields the beam peak radiation would have to be declared. A sample declaration is shown in Table 5.1.3-2.

Figure 5.1.3-7: Illustration of low UL power test case along TX BP direction of sample DUT utilizing white-box approach.

Table 5.1.3-2: Sample Vendor Declaration for white-box approach supporting low UL power test cases

| Antenna Panel (yielding TX beam peak radiation) | Phase-centre offset from geometric centre of DUT |
| --- | --- |
|  | (xoff, yoff, zoff) |

Two different black&white-box approaches could be further considered, i.e.,

-	Extensive Black&white-box approach: When the NF methodology is used for spherical coverage test cases and for beam peak searches, all active antenna locations are declared together with the angular ranges (theta, phi) each active antenna performs best (when compared to the remaining antenna panels, i.e., the vendor declaration is as outlined in Table 5.1.3-1. Very much similar to the white-box approach with the only difference that the geometric centre of DUT is aligned with the centre of QZ.

-	Black&white box: When the NF methodology is used only for EIS based high DL power or EIRP/TRP based low UL power test cases, only the antenna location of the antenna that yields the beam peak needs to be declared, i.e., the vendor declaration is as outlined in Table 5.1.3-2. The geometric centre of DUT is aligned with the centre of QZ.

For test cases focused only on the for EIS based high DL power or EIRP/TRP based low UL power test cases, the key differences are illustrated in Figure 5.1.3-8 for the black-box approach (left), black&white-box approach (centre), and the white-box approach (right). While the black-box approach requires local searches to determine the NF test direction, the need for local searches for the black&white-box approach is FFS. No local search is necessary for the white-box approach.

For the spherical coverage test cases or the beam peak searches, the extensive black&white-box approach is further outlined in Figure 5.1.3-9. On the other hand, the black-box approach is outlined in Figure 5.1.3-4 while the white-box approach is outlined in Figure 5.1.3-6.

Figure 5.1.3-8: Illustration of black-box approach (left), black&white-box approach (centre), and the white-box approach (right) for the low-UL power test case.

Figure 5.1.3-9: Illustration of beam peak search or beam peak search of sample DUT utilizing extensive black&white-box approach

For white box testing, the minimum radius of the NF probe antenna from the centre of the quiet zone generally must exceed the maximum diameter of the device, as illustrated in Figure 5.1.3-10, to prevent interference of the near field scanning probe with the DUT. While this requirement of the NF range length having to exceed the maximum diameter of the DUT is generally applicable to TRP where the NF Probe antenna needs to perform a full 3D scan around the DUT, this could very well be applicable to single-directional measurements as well, as illustrated in Figure 5.1.3-10 using a PC1 CPE as an example. Similar restrictions apply when testing using ETC enclosures surrounding the DUT.

Figure 5.1.3-10: Illustration of min. Range length of NF Systems when applying white box testing

Figure 5.1.3-11: Illustration of min. Range length for NF Systems using PC1 CPE as example.

The corresponding FF and NF min. range lengths are tabulated for selected FR2 frequencies in Table 5.1.3-3 for PC3 devices with fixed D=5cm.

Table 5.1.3-3: Minimum FF and NF Range Lengths for black box and white box conditions for PC3 devices

| f [GHz] | Antenna Config. 1 and 2 - BLACK BOX - (PC3 Devices: D=5cm) |  | Antenna Config. 1 and 2 - WHITE BOX - (PC3 Devices: D=5cm) |  |
| --- | --- | --- | --- | --- |
|  | Min. FF Range Length [m] DDUT2-D2+2D2λ | Min. NF Range Length [m] DDUT2-D2+0.62D3λ | Min. FF Range Length [m] max2D2λ,DDUT-D2 | Min. NF Range Length [m] max0.62D3λ,DDUT-D2 |
| 24.25 | 0.53 | 0.19 | 0.40 | 0.28 |
| 30 | 0.63 | 0.19 | 0.50 | 0.28 |
| 40 | 0.79 | 0.21 | 0.67 | 0.28 |
| 43.5 | 0.85 | 0.21 | 0.73 | 0.28 |
| 52.6 | 1.00 | 0.22 | 0.88 | 0.28 |

Table 5.1.3-4 summarizes the path loss comparison between “white box” and “black box” configuration across IFF/DFF and NF system types.

Table 5.1.3-4: Path loss comparison between “white box” and “black box” configuration

| f (GHz) | Antenna Config. 1, 2, and 3 - BLACK BOX - (PC3 Devices: D=5cm) |  | Antenna Config. 1 and 2 - WHITE BOX - (PC3 Devices: D=5cm) |  |
| --- | --- | --- | --- | --- |
|  | IFF/DFF | NF | DFF | NF |
|  | Path Loss with 1m range length | Path Loss with 0.22m range length | Path Loss with 0.88m range length | Path Loss with 0.28m range length |
| 24.25 | 60.16 | 46.86 | 59.01 | 48.93 |
| 30 | 62.01 | 48.71 | 60.85 | 50.78 |
| 40 | 64.51 | 51.21 | 63.35 | 53.28 |
| 43.5 | 65.24 | 51.94 | 64.08 | 54.00 |
| 52.6 | 66.89 | 53.59 | 65.73 | 55.65 |

Based on the analysis shown in Table 5.1.3-4, it can be concluded that a “white box” is not deemed a feasible enhancement of the methodology.

Additionally, since the beam peak searches and the spherical coverage test cases are not part of the low UL/high DL power test cases and given the complexity of the vendor declaration of the extensive black&white-box approach, it can be concluded that the extensive black&white-box approach is not deemed a feasible enhancement of the methodology.

### 5.1.4	Permitted Methodologies: CFFNF, CFFDNF and CFFdeltaNF

Both methodologies have in common that a FF probe, e.g., reflector & feed probe from the IFF methodology, is used for the test cases that are not considered low UL power/high DL power. This FF probe is used for the low UL/high DL power test cases to steer and lock the beam in the known FF direction before the NF measurements are performed with a NF probe that exhibits much lower free-space path losses. An example test setup of such a hybrid system is shown in Figure 5.1.4-1.

The main differences between the three measurement approaches are outlined in Table 5.1.4-1.

Table 5.1.4-1: Main differences between CFFDNF and CFFNF measurement approaches

| ► Methodology ► ▼ Test Approach ▼ | CFFDNF | CFFNF | CFFdeltaNF |
| --- | --- | --- | --- |
| Black Box | N/A (Note 1) | Wide local search at initial radius r1, narrow local searches at radii r2, r3, i.e., multiple NF measurements at r1, r2, and r3 | Local search at fixed radius, reference measurement at FF and NF to obtain the relative correction factor ∆NFtoFF, single NF measurement |
| Black&white-box | Single NF measurement or local search at r1 | Single NF measurements at r1, and r2 | N/A |
| Note 1: This can be revised whenever empirical methods to determine the offset location or other methods are shown feasible. |  |  |  |

Figure 5.1.4-1: Hybrid NF/(I)FF test setup suitable for NF measurements

In a NF system, the NF beam peak direction for an offset antenna is not necessarily the same as the FF beam peak direction; however, the knowledge of the antenna phase centre offset, i.e., black&white-box approach, can be leveraged to measure at the NF beam peak direction as illustrated in 5.1.4-2. The knowledge of the offset together with the probe antenna pattern will allow the calculation of the optimized DUT orientation to optimize the NF measurement. The beam peak direction in the NF can either be calculated or determined via a local search.

Figure 5.1.4-2: Illustration of NF testing utilizing the black&white-box approach.

To guarantee that the correct beam is measured for when the black&white-box measurement approach is applied, the CFF(D)NF approach utilizes a FF probe that allows the UE to select the proper beam in the known beam peak direction. A beam lock activation via the UBF makes sure that the UE no longer changes its antenna pattern when the NF measurement probe is used to perform the measurements with significantly reduced free-space path losses compared to existing IFF systems.

The CFFDNF approach and test steps for EIRP/EIS/TRP follow those for IFF/DFF outlined in Annex K [6] with the exception that the minimum range length is reduced. The minimum number of TRP grid points, required grid point spacing, and the effect of compensation of the declared/known offset is further outlined in this this clause.

#### 5.1.4.1	Asymptotic Expansion Approach for CFFNF

The asymptotic expansion approach for CFFNF that is using measurements

-	at three different radii including local searches around the FF beam peak direction (black-box approach) or

-	at two different radii without local searches as the NF beam peak direction can be calculated (black&white-box approach)

is primarily meant to estimate the FF EIRP/EIS at the beam peak with NF EIRP/EIS measurements performed in the radiative NF instead of the reactive NF. This expansion approach is not suitable to determine the FF pattern given the required overhead with the measurements at multiple radii.

The asymptotic expansion approach is based on the superposition approach

Signalθ,ϕ∝k=1Ny⋅NzxkFkθk,ϕkFpαk,βkλ4πdkej2πdkλ

with

-	Signal(𝜃, ϕ): signal at measurement grid point (𝜃, ϕ, 𝑟)

-	xk: complex coefficient for kth antenna array element

-	Fk: field pattern for kth antenna element

-	Fp: field pattern for probe antenna

-	(𝜃k, ϕk): EL and AZ angles of kth antenna element with respect to measurement grid point (𝜃, ϕ, 𝑟)

-	(𝛼k, 𝛽k): EL and AZ angles of probe antenna with respect to the kth antenna element

-	λ: wavelength

-	dk: distance between kth antenna element to measurement grid point (𝜃, ϕ, 𝑟)

-	r: range length/measurement distance between centre of QZ and probe antenna

-	λ4πdk: amplitude loss caused by propagation on length of dk

-	2πdkλ: the phase variation caused by propagation on length of dk

The estimate of the FF EIRP based on NF measurements, p(d), can be expressed as follows

EIRPdf=pd1+ d1df∂p∂d d∆d

with

p∝Signal⋅d2

where EIRP(df) is the estimated EIRP in the far-field and df is the distance between the phase centre of the antenna array and the far-field beam peak at range length r (in the FF: r ~ df) while p(d1) is the measured power with the probe antenna at a near-field distance d1 (measured from the phase centre of the antenna array to probe antenna), ∂p/∂d is the derivative of power p to distance d. Because the near-field distance d1 is unknown for the black box approach, measurements of the EIRP at multiple measurement distances are needed to determine the parameters of the ∂p/∂d derivative and the first near-field distance d1.

Following the implementation of the CFFNF simulations and an in-depth analysis of the derivative of the superposition expression above, the approximation of normalized power to distance ∂p/∂d was determined to be

∂p∂d=b1d-3

which yields dependency of normalized power to d of

pd=-b12d-2+b2

This approximation in turn corresponds to a power (field) dependence in the NF of 1/r4 (1/r2) which is commonly observed in the radiative NF. CST simulations were used to further support this formulation. Here, an 8x2 antenna array, placed at (0,0,0), was evaluated. Two measurements at r1 and r2 were used to estimate the asymptotic expansion coefficients bi; this allowed to estimate the power/EIRP at various distances which was can be compared with actual CST simulations at those distances. The ∂p∂d=b1d-3 behaviour and the normalized power (compensating the path loss) estimated from two sets of two measurements at (r1, r2) of (7.5cm, 9.5cm) and (20cm, 22cm) simulated at various distances (>r2) are shown in Figure 5.1.4.1-1.

Figure 5.1.4.1-1: Normalized power (left) and power derivative (right) and as a function of transformed distance. The NF evaluations were performed at r1=7.5cm, r2=8.5cm, r3=9.5cm (top) and at r1=20cm, r2=21cm, r3=22cm (bottom)

These curves show that the ∂p∂d=b1d-3 ; p=-b12d-2+b2 asymptotic expansion formulation yields good estimates of EIRP/EIS based on measurements in the radiative NF.

#### 5.1.4.2	Test Procedures for CFFDNF, CFFNF and CFFdeltaNF

The appropriate test steps required for NF testing based on the CFFDNF approach of DUTs with known phase-centre offsets (black&white-box) are illustrated in Figure 5.1.4.2-1.

Figure 5.1.4.2-1: Test Steps for CFFDNF testing of DUTs with known antenna phase centre offset (black&white-box approach).

For the CFFNF methodology that supports both the black-box and the black&white-box approach, the initial test steps are the same as steps 1-3 in Figure 5.1.4.2-1. The test steps for the NF measurement portion of the black-box approach are further outlined in Figure 5.1.4.2-2 while the NF test steps for the black&white-box approach are outlined in Figure 5.1.4.2-3. The diagrams on the right of Figure 5.1.4.2-2 illustrate the different local searches required for the measurements at each of the three radii. The measurements at the very first radius r1 require a wide sector of grid points around the known FF beam peak direction big enough so that the local/NF beam peak is captured properly. For the initial local search at r1=20cm, the width of the sector is about ±40o which can be covered using coarse and fine scans to further reduce the number of points. On the other hand, the sector of grid points for measurements at radius r2 and r3 can be significantly smaller as only a small region around the local NF beam peak found at r1 is needed.

Figure 5.1.4.2-2: Illustration of the additional CFFNF test steps with asymptotic expansion transform utilizing the black-box approach.

Figure 5.1.4.2-3: Illustration of the additional CFFNF test steps with asymptotic expansion transform utilizing the black&white-box approach.

For the CFFdeltaNF methodology that supports the black-box approach, the initial test steps are the same as steps 1-3 in Figure 5.1.4.2-1. The additional test steps for the NF relative and final measurement portion of the black-box approach are further outlined in Figure 5.1.4.2-4. The diagrams on the right of Figure 5.1.4.2-4 illustrate the local search required for the measurements in NF. At r=35cm, the width of the sector is about ±20o which can be covered using coarse and fine scans to further reduce the number of points.

Figure 5.1.4.2-4: Illustration of the additional CFFdeltaNF test steps utilizing the black-box approach.

The sample sequence of test steps for the CFFDNF test methodology with the black&white-box approach is as follows (aligned in principle with Clause K.1.3 of [6]):

1.	Connect the SS (System Simulator) with the DUT through the FF measurement antenna with polarization reference PolLink to form the TX beam towards the FF TX beam peak direction. Allow at least BEAM_SELECT_WAIT_TIME for the UE TX beam selection to complete.

2.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 clause 4.9.2 using condition Tx only.

3.	If necessary, switch the connection of the SS from the FF probe to the NF probe at range length r1 with polarization reference PolLink. Position the device so that the NF probe antenna is pointed towards the NF TX beam peak direction (determined from the antenna offset, range length, and FF beam peak direction). The range length is left up to system implementation.

4.	Measure the mean power Pmeas(PolMeas= PolLink) of the modulated signal arriving at the power measurement equipment (such as a spectrum analyser, power meter, or gNB emulator).

5.	Calculate the EIRP(PolMeas= PolLink from the measured power Pmeas(PolMeas= PolLink

a.	adding the calibrated composite loss of the entire transmission path between the TE and the centre of QZ (displaced from measurement probe by r), LEIRP,θ

b.	compensating the actual measurement distance d (between the centre of the array to the measurement probe) using term 20 log10(d/r)

c.	applying the probe antenna gain NF correction

6.	Measure the mean power Pmeas(PolMeas= PolLink) of the modulated signal arriving at the power measurement equipment (such as a spectrum analyser, power meter, or gNB emulator).

7.	Calculate the EIRP(PolMeas= PolLink from the measured power Pmeas(PolMeas= PolLink

a.	adding the calibrated composite loss of the entire transmission path between the TE and the centre of QZ (displaced from measurement probe by r), LEIRP,ϕ

b.	compensating the actual measurement distance d (between the centre of the array to the measurement probe) using term 20 log10(d/r)

c.	applying the probe antenna gain NF correction

8.	Calculate the resulting “total EIRP(PolLink)”, for the chosen PolLink of oras follows:

total EIRP (PolLink = EIRP(PolMeas= PolLink + EIRP(PolMeas= PolLink

9.	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 clause 4.9.3.

The sample sequence of test steps for the CFFNF test methodology with the black&white-box approach is as follows (aligned in principle with Clause K.1.3 of [6]):

1.	Connect the SS (System Simulator) with the DUT through the FF measurement antenna with polarization reference PolLink to form the TX beam towards the FF TX beam peak direction. Allow at least BEAM_SELECT_WAIT_TIME for the UE TX beam selection to complete.

2.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 clause 4.9.2 using condition Tx only.

3.	If necessary, switch the connection of the SS from the FF probe to the NF probe at range length r=r1 /at measurement distance d=d1 (between the centre of the array to the measurement probe) with polarization reference PolLink.

4.	Position the device so that the NF probe antenna is pointed towards the NF TX beam peak direction (determined from the antenna offset, range length r, and FF beam peak direction). The range length r is left up to system implementation.

5.	Measure N averages of the mean power Pmeas(d, PolMeas= PolLink) of the modulated signal arriving at the power measurement equipment (such as a spectrum analyser, power meter, or gNB emulator). The number of averages is left up to system implementation.

6.	Calculate the normalized NF power p(d, PolMeas= PolLink from the measured power Pmeas(d, PolMeas= PolLink

a.	adding the calibrated composite loss of the entire transmission path between the TE and the centre of QZ (displaced from measurement probe by r=r1), LEIRP,θ

b.	compensating the actual measurement distance d using term 20 log10(d/r)

c.	applying the probe antenna gain NF correction

7.	Measure N averages of the mean power Pmeas(d, PolMeas= PolLink) of the modulated signal arriving at the power measurement equipment (such as a spectrum analyser, power meter, or gNB emulator). The number of averages is left up to system implementation.

8.	Calculate the normalized NF power p(d, PolMeas= PolLink from the measured power Pmeas(d, PolMeas= PolLink

a.	adding the calibrated composite loss of the entire transmission path between the TE and the centre of QZ (displaced from measurement probe by r), LEIRP,ϕ

b.	compensating the actual measurement distance d (between the centre of the array to the measurement probe) using term 20 log10(d/r)

c.	applying the probe antenna gain NF correction

9.	Repeat Steps 4-8 for r=r2 and d=d2

10.	Calculate the total normalized NF power for the chosen PolLink of oras follows:

p(di = p(di, PolMeas= PolLink + p(di, PolMeas= PolLink) with i={1,2}

11.	Based on the selected asymptotic expansion formulation, determine the “total FF EIRP(PolLink)” from the two total normalized NF power measurement measurements, p(d1) and p(d2). For an asymptotic expansion formulation of

pd=-b12d-2+b2

The resulting “total FF EIRP(PolLink)”, for the chosen PolLink of or is calculated as follows

total FF EIRPPolLink=d12pd1-d22pd2d12-d22

12.	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 clause 4.9.3.

The sample sequence of test steps for the CFFNF test methodology with the black-box approach is as follows (aligned in principle with Clause K.1.3 of [6]):

1.	Connect the SS (System Simulator) with the DUT through the FF measurement antenna with polarization reference PolLink to form the TX beam towards the FF TX beam peak direction. Allow at least BEAM_SELECT_WAIT_TIME for the UE TX beam selection to complete.

2.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 clause 4.9.2 using condition Tx only.

3.	If necessary, switch the connection of the SS from the FF probe to the NF probe at range length r1 with polarization reference PolLink. Position the device so that that NF probe antenna is pointed towards the FF TX beam peak direction. The range length r1 is left up to system implementation.

4.	Perform a NF BP search on a sector around the FF BP direction at radius r=r1, which could determine K (≥1) possible NF BP directions and corresponding antenna array phase centre positions and thus distances between the antenna array and the measurement probe, d1,k. Based on the NF BP directions and antenna array phase centre positions, the corresponding NF BP directions at radius r=r2 and r=r3 can be determined. Details including the range lengths r2 and r3 are left up to system implementation.

5.	Measure N averages of the mean power Pmeas,k(di,k, PolMeas= PolLink), k={1,2,…, K} of the modulated signal arriving at the power measurement equipment (such as a spectrum analyser, power meter, or gNB emulator) at each of the K possible NF BP direction at radius r=ri with i={1,2,3} determined in Step 4. The number of averages is left up to system implementation.

6.	Calculate the normalized NF power pmeas,k(di,k, PolMeas= PolLink, k={1,2,…, K}, from the measured power Pmeas,k(di,k, PolMeas= PolLink), k={1,2,…,K}, at radius r=ri with i={1,2,3}

a.	adding the calibrated composite loss of the entire transmission path between the TE and the centre of QZ (displaced from measurement probe by r=ri), LEIRP,θ

b.	compensating the actual measurement distance d=di (between the centre of the array to the measurement probe) using term 20 log10(di,k/ri)

c.	applying the probe antenna gain NF correction

7.	Measure N averages of the mean power Pmeas,k(di,k, PolMeas= PolLink), k={1,2,…,K}, of the modulated signal arriving at the power measurement equipment (such as a spectrum analyser, power meter, or gNB emulator) at each of the possible NF BP direction at radius r=ri with i={1,2,3} determined in Step 4. The number of averages is left up to system implementation.

8.	Calculate the normalized NF power pmeas,k(di,k, PolMeas= PolLink k={1,2,…, K},  from the measured power Pmeas,k(di,k, PolMeas= PolLink), k={1,2,…, K}, at radius r=ri with i={1,2,3}

a.	adding the calibrated composite loss of the entire transmission path between the TE and the centre of QZ (displaced from measurement probe by r=ri), LEIRP,

b.	compensating the actual measurement distance d=di (between the centre of the array to the measurement probe) using term 20 log10(di,k/ri)

c.	applying the probe antenna gain NF correction

9.	Calculate the total normalized NF power for each of possible NF BP directions with the chosen PolLink of oras follows:

pmeas,k(di = pmeas(di,k, PolMeas= PolLink + pmeas(di,k, PolMeas= PolLink) with k={1,2,…,K} and  i={1,2,3}

10.	For each of K possible NF BP directions, based on the pmeas,k, k={1,2,…,K}, results at r=r1, r=r2, and r=r3 perform a linear fitting to determine far-field normalized power “total FF EIRPk(PolLink)” and fitting error errk, based on the selected expansion formulation, e.g.,

pd=-b12d-2+b2

11.	Determine the final NF BP direction by choosing the NF BP direction with minimum fitting error kBP=minkerrk. Details of this step are left up to system implementation.

12.	Based on the selected NF BP direction in Step 11, the resulting “total FF EIRP(PolLink)”, for the chosen PolLink of or is determined as totalFFEIRPkBPPolLink

13.	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 clause 4.9.3.

The sample sequence of test steps for the CFFdeltaNF test methodology with the black-box approach is as follows (aligned in principle with Clause K.1.3 of [6]):

1.	Connect the SS (System Simulator) with the DUT through the FF measurement antenna with polarization reference PolLink to form the TX beam towards the FF TX beam peak direction. Allow at least BEAM_SELECT_WAIT_TIME for the UE TX beam selection to complete.

2.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 clause 4.9.2 using condition Tx only.

3.	Measure the mean power Pmeas(PolMeas= PolLink) and Pmeas(PolMeas=ϕ PolLink) of the modulated signal arriving at the power measurement equipment (such as a spectrum analyser, power meter, or gNB emulator) through the FF measurement antenna for the reference test case without testability issue (e.g. Peak EIRP).

4.	If necessary, switch the connection of the SS from the FF probe to the NF probe at range length r1 with polarization reference PolLink. The range length is left up to system implementation.

5.	Position the device so that the NF probe antenna is pointed towards the FF TX beam peak direction and perform a NF BP search on a sector around the FF BP direction at range length r1 to determine the NF Tx beam peak direction.

6.	Measure the mean power Pmeas(PolMeas= PolLink) and Pmeas(PolMeas=ϕ PolLink) of the modulated signal arriving at the power measurement equipment (such as a spectrum analyser, power meter, or gNB emulator) through the NF measurement antenna for the reference test case without testability issue (e.g. Peak EIRP) at the NF beam peak direction.

7.	Calculate the NF correction factor ∆NFtoFF for each polarization as the difference between the measured mean power and the composite loss of the entire transmission path through the FF and NF measurement antennas:

∆NFtoFF, θ=(Pmeas, FF,θ-Pmeas, NF,θ)+LFF,θ-LNF,θ

∆NFtoFF, ϕ=(Pmeas, FF,ϕ-Pmeas, NF,ϕ)+LFF,θ-LNF,ϕ

Note: this correction factor ∆NFtoFF include all effects due to the usage of a NF probe (i.e. DUT antenna location displacement from center, probe antenna pattern and near-field interaction between probe antenna and DUT antenna)

8.	If necessary, calculate the NF correction factor ∆NFtoFF for each polarization at the correct test frequency for the low UL power test case to include the additional effect of the system frequency response:

∆NFtoFF, θf=∆NFtoFF, θfref+LFF, θf-LNF, θf

∆NFtoFF, ϕf=∆NFtoFF, ϕfref+LFF, ϕf-LNF, ϕf

9.	Measure the mean power Pmeas(PolMeas= PolLink) of the modulated signal arriving at the power measurement equipment (such as a spectrum analyser, power meter, or gNB emulator) through the NF measurement antenna for the low UL power test case at the NF beam peak direction.

10.	Calculate the EIRP(PolMeas= PolLink from the measured power Pmeas(PolMeas= PolLinkby adding the composite correction factor between FF and DNF ∆NFtoFF, θ and the frequency to the measured power Pmeas(PolMeas= PolLink).

11.	Measure the mean power Pmeas(PolMeas= PolLink) of the modulated signal arriving at the power measurement equipment (such as a spectrum analyser, power meter, or gNB emulator) through the NF measurement antenna for the low UL power test case at the NF beam peak direction.

12.	Calculate the EIRP(PolMeas= PolLink from the measured power Pmeas(PolMeas= PolLinkby adding the composite correction factor between FF and DNF ∆NFtoFF, ϕ and the frequency to the measured power Pmeas(PolMeas=ϕ PolLink).

13.	Calculate the resulting “total EIRP(PolLink)”, for the chosen PolLink of oras follows:

total EIRP (PolLink = EIRP(PolMeas= PolLink + EIRP(PolMeas= PolLink

14.	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 clause 4.9.3.

#### 5.1.4.3	Simulation Assumptions

The simulation assumptions for the CFFDNF, CFFNF and CFFdeltaNF methodologies are summarized in Table 5.1.4.3-1.

Table 5.1.4.3-1: Simulation assumptions for CFFDNF, CFFNF and CFFdeltaNF simulations

| Parameter | Value(s)/Assumptions | Comment |
| --- | --- | --- |
| Methodology | CFFDNF: with black&white-box approach CFFNF: with black-box and black&white-box approach CFFdeltaNF: with black-box approach |  |
| Simulation Frequency [GHz] | 28 (others are not precluded) |  |
| UE Antenna Array Configuration | PC3: 8x2 and 4x1 PC1: 12x12 |  |
| Beam Steering Assumptions | N/A | Not needed for combined FF and NF methods as beam peak searches and spherical coverage measurements are based on FF probe |
| HPBW of Individual Array Element | 90o/90o |  |
| Offsets of Active Array Panel | PC3 8x2&4x1: 0 ≤ xoffset ≤ 12.5cm -12.5cm ≤ yoffset ≤ 12.5cm -12.5cm ≤ zoffset ≤ 12.5cm (The maximum radial offset cannot exceed 12.5cm) PC1 12x12: 0 ≤ xoffset ≤ 10 cm -10cm ≤ yoffset ≤ 10cm -10cm ≤ zoffset ≤ 10cm (The maximum radial offset cannot exceed 10cm) | Offsets should be picked randomly (for uniform distribution) Min of 500 offsets selected randomly with uniform distribution |
| Path Loss Correction | Compensation of antenna array offset for black&white-box approaches and the CFFNF black-box approach. N/A for CFFdeltaNF black-box approach | Path loss applied to the EIRP measurements is not referenced to the centre of QZ but to the phase centre of the active antenna array |
| NF Measurement Direction | Determined theoretically from range length, FF BP direction, and array offsets for black&white-box approach. Determined with local search for black-box approach. |  |
| Probe antenna pattern/gain compensation | With compensation (uniform pattern assumed in simulations) Without compensation (typical horn pattern with ~50o HPBW pattern applied) | N/A for CFFdeltaNF |
| Tool Used for Simulations | Matlab or EM simulator |  |
| Range Lengths | 20cm, 25cm, 30cm, 35cm, 40cm, 45cm, 20m |  |

#### 5.1.4.4	Simulation results for CFFDNF

The main intention of this clause is to estimate the measurement uncertainties of EIRP measurements performed in the NF at various range lengths. Since the beam peak search and spherical coverage analyses are performed with the FF probe, beam steering assumptions are not required here. The definitions of offsets (xoffset/yoffset/zoffset), maximum offsets (≤12.5cm for PC3 and ≤10cm for PC1), array configurations (PC3: 8x2 and 4x1, PC1: 12x12), and range lengths are summarized in Table 5.1.4.3-1.

The FF 8x2 and 4x1 array patterns with the 90o/90o HPBW assumption are shown in Figure 5.1.4.4-1.

Figure 5.1.4.4-1: FF antenna pattern with 90o/90o HPBW for 8x2 antenna array configuration (left) and 4x1 antenna array configuration (right).

The pattern simulations assume superpositions of individual, single-element far-field antenna patterns; this approach requires that the NF of Ny x Nz antenna array is well in the FF of the single-element antenna. The EIRP simulations were performed using Matlab and CST.

The simulations assume that the FF beam peak direction of the DUT is known for the sample DUT considered.

For the statistical analyses using Matlab, a total of 100,000 simulations with random and uniformly spaced offsets were performed. These offsets were varied between 0 to 12.5cm (10cm) in x and from -12.5cm (-10cm) to 12.5cm (10cm) in y and z for PC3 (PC1) while making sure that the maximum radial offset cannot exceed 12.5cm (10cm). The offsets in x were limited to positive values since it is assumed that the front antenna of the DUT is always in the upper hemisphere since the geometric centre of the device is aligned with the centre of the QZ. A sample set of 100,000 random offsets is visualized in Figure 5.1.4.4-2. Histograms of the respective offset radii, and offsets in x, y, and z are shown in Figure 5.1.4.4-3.

Figure 5.1.4.4-2: Illustration of 100,000 random offsets uniformly spaced within 12.5cm in a single hemisphere.

Figure 5.1.4.4-3: Histograms of 100,000 random offsets uniformly spaced within 12.5cm in a single hemisphere.

Since each of the offsets are known/declared, the offset can be properly compensated, i.e., the pathloss applied to the EIRP measurements is not referenced to the centre of QZ but to the phase centre of the active antenna array. The results in this clause focus only on the EIRP results after the path loss with respect to the offset antenna array was compensated.

Additionally, for the best/optimized measurement uncertainties, the probe antenna pattern/gain must be compensated since the array offsets can result in the NF beam peak to be observed from directions with large deviations from the peak gain direction of the measurement probe/horn as illustrated in Figure 5.1.4-2. In the simulations, probe pattern/gain compensation can be modelled in the simplest approximation by assuming an omnidirectional pattern of the probe. To quantify the effect of not compensating the probe antenna pattern, this section will present measurement uncertainties for a typical horn antenna. For these simulations, a symmetric pattern of a horn antenna with ~50o HPBW pattern is assumed as plotted in Figure 5.1.4.4-4, which was obtained using the following Matlab commands:

ProbeTheta=-180:1:180;

HPBW=50;

ProbePattern_norm=-12*(ProbeTheta/HPBW);

Figure 5.1.4.4-4: Assumed measurement probe antenna pattern.

The NF beam peak direction, illustrated in Figure 5.1.4-2, was calculated using the known FF beam peak direction, the offset of the antenna array, and the range length.

A histogram of the 100k EIRP simulations for 4 different NF range lengths (20cm, 25cm, 30cm, 45cm) and the 20m FF range lengths is shown in Figure 5.1.4.4-5 and the statistical results of these simulations are tabulated in Table 5.1.4.4-1. These results assume that the antenna array offsets and the probe pattern/gain were compensated.

Figure 5.1.4.4-5: Histogram of 100,000 EIRP CFFDNF simulations based on black&white-box approach with random 8x2 antenna array offsets uniformly spaced within 12.5cm in a single hemisphere. The antenna array offsets and the probe pattern/gain were compensated

Table 5.1.4.4-1: Statistical results of 100,000 EIRP CFFDNF simulations based on black&white-box approach with random 8x2 antenna array offsets uniformly spaced within 12.5cm in a single hemisphere. The antenna array offsets and the probe pattern/gain were compensated

| Range Length [m] | Max-Min EIRP [dB] | Max EIRP Error w.r.t. FF [dB] | ¦Mean EIRP Error¦ w.r.t. FF [dB] | Std. Dev of EIRP [dB] |
| --- | --- | --- | --- | --- |
| 0.2 | 1.17 | 1.36 | 0.48 | 0.22 |
| 0.25 | 0.37 | 0.50 | 0.23 | 0.08 |
| 0.3 | 0.17 | 0.26 | 0.14 | 0.04 |
| 0.35 | 0.09 | 0.16 | 0.09 | 0.02 |
| 0.4 | 0.06 | 0.10 | 0.07 | 0.01 |
| 0.45 | 0.04 | 0.07 | 0.05 | 0.01 |
| 20 | 0.00 | 0.00 | 0.00 | 0.00 |

Figure 5.1.4.4-6 illustrates which simulations for the 20cm range length result in the minimum and maximum EIRPs after antenna offset and probe pattern compensation.

Figure 5.1.4.4-6: Illustration of simulations for the 20cm range length with 8x2 antenna configuration resulting in smallest (left) and largest (right) EIRPs after antenna offset and probe pattern compensation.

When the antenna array offset is towards the probe antenna, shown in the left plot of Figure 5.1.4.4-6, the EIRP without the offset compensation is very high (20.7dBm in this example); however, the offset compensation, i.e., applying the pathloss between the probe antenna and the active antenna array, helps to significantly improve the EIRP measurement uncertainty with respect to the EIRP measured in the FF.

Once the array offsets and the probe antenna pattern are compensated in the NF with CFFDNF methodology assuming the black&white-box approach, almost insignificant measurement uncertainties for PC3 devices can be observed at 45cm. At distances less than 45cm, measurement uncertainties must be taken into account.

When the probe pattern/gain is not compensated, a much larger variation of the measured EIRP results is expected due to the large off broadside directions of the antenna panels from the probe antenna, as illustrated schematically in Figure 5.1.4-2. This is further quantified in Figure 5.1.4.4-6 and in Table 5.1.4.4-2 for the same simulations. These results assume that the antenna array offsets are compensated while the probe pattern/gain were not compensated, i.e., the pattern in Figure 5.1.4.4-4 was applied to the simulations.

Figure 5.1.4.4-7: Histogram of 100,000 EIRP CFFDNF simulations based on black&white-box approach with random 8x2 antenna array offsets uniformly spaced within 12.5cm in a single hemisphere. The antenna array offsets were compensated while the probe pattern/gain was not compensated

Table 5.1.4.4-2: Statistical results of 100,000 EIRP CFFDNF simulations based on black&white-box approach with random 8x2 antenna array offsets uniformly spaced within 12.5cm in a single hemisphere. The antenna array offsets were compensated while the probe pattern/gain was not compensated.

| Range Length [m] | Max-Min EIRP at (90,0) [dB] | Max EIRP Error w.r.t. FF [dB] | ¦Mean EIRP Error¦ w.r.t. FF [dB] | Std. Dev of EIRP at (90,0) [dB] |
| --- | --- | --- | --- | --- |
| 0.2 | 7.31 | 7.51 | 3.15 | 1.82 |
| 0.25 | 4.36 | 4.49 | 1.89 | 1.11 |
| 0.3 | 2.93 | 3.02 | 1.27 | 0.75 |
| 0.35 | 2.11 | 2.18 | 0.92 | 0.54 |
| 0.4 | 1.60 | 1.65 | 0.69 | 0.41 |
| 0.45 | 1.25 | 1.29 | 0.54 | 0.32 |
| 20 | 0.00 | 0.00 | 0.00 | 0.00 |

The latter results clearly demonstrate that when performing measurements in the NF with CFFDNF methodology assuming the black&white-box approach, the probe antenna pattern must be compensated.

A study to determine whether 1k or even 250 offset simulations are sufficient for the MU results, a comparison of 100k vs 1k vs 250 offset simulations was made. The visualization of 100k vs 1k random offsets is illustrated in Figure 5.1.4.4-8. Clearly, the 100k offsets are uniformly distributed in the hemisphere while the random 250 and 1k offsets are distributed rather sparsely.

Figure 5.1.4.4-8: Illustration of 100k (left) vs 1k (middle) vs 250 (right) offsets.

The results summarizing the different simulations are tabulated in Table 5.1.4.4-3. The results show that regardless of range length and antenna configuration, the difference in mean error and standard deviation is almost insignificant.

Table 5.1.4.4-3: Statistical results of 100k vs 1k vs 250 EIRP CFFDNF offset simulations based on black&white-box approach with random antenna array offsets uniformly spaced within 12.5cm (PC3)/10cm (PC1) in a single hemisphere. The antenna array offsets were compensated.

| Antenna Configuration | Number of Offsets | Range Length [m] | Mean EIRP Error w.r.t. FF [dB] | Std. Dev of EIRP at NF BP [dB] |
| --- | --- | --- | --- | --- |
| 8x2 | 100k | 0.2 | 0.48 | 0.22 |
|  |  | 0.25 | 0.23 | 0.08 |
|  |  | 0.3 | 0.14 | 0.04 |
|  |  | 0.35 | 0.09 | 0.02 |
|  |  | 0.4 | 0.07 | 0.01 |
|  |  | 0.45 | 0.05 | 0.01 |
|  |  | 20 | 0.00 | 0.00 |
| 8x2 | 1k | 0.2 | 0.48 | 0.21 |
|  |  | 0.25 | 0.23 | 0.07 |
|  |  | 0.3 | 0.14 | 0.03 |
|  |  | 0.35 | 0.09 | 0.02 |
|  |  | 0.4 | 0.07 | 0.01 |
|  |  | 0.45 | 0.05 | 0.01 |
|  |  | 20 | 0.00 | 0.00 |
| 8x2 | 250 | 0.2 | 0.49 | 0.23 |
|  |  | 0.25 | 0.23 | 0.08 |
|  |  | 0.3 | 0.14 | 0.04 |
|  |  | 0.35 | 0.09 | 0.02 |
|  |  | 0.4 | 0.07 | 0.01 |
|  |  | 0.45 | 0.05 | 0.01 |
|  |  | 20 | 0.00 | 0.00 |
| 12x12 | 100k | 0.2 | 3.41 | 1.09 |
|  |  | 0.25 | 1.84 | 0.44 |
|  |  | 0.3 | 1.16 | 0.22 |
|  |  | 0.35 | 0.80 | 0.13 |
|  |  | 0.4 | 0.59 | 0.08 |
|  |  | 0.45 | 0.45 | 0.05 |
|  |  | 20 | 0.00 | 0.00 |
| 12x12 | 1k | 0.2 | 3.43 | 1.10 |
|  |  | 0.25 | 1.85 | 0.44 |
|  |  | 0.3 | 1.17 | 0.22 |
|  |  | 0.35 | 0.81 | 0.13 |
|  |  | 0.4 | 0.59 | 0.08 |
|  |  | 0.45 | 0.45 | 0.05 |
|  |  | 20 | 0.00 | 0.00 |
| 12x12 | 250 | 0.2 | 3.47 | 1.13 |
|  |  | 0.25 | 1.87 | 0.45 |
|  |  | 0.3 | 1.18 | 0.23 |
|  |  | 0.35 | 0.81 | 0.13 |
|  |  | 0.4 | 0.60 | 0.08 |
|  |  | 0.45 | 0.46 | 0.06 |
|  |  | 20 | 0.00 | 0.00 |

Another investigation focused on the MUs at two different frequencies at opposite ends of FR2, i.e., 28GHz and 49GHz using the same fixed range lengths. The simulation results are tabulated in Table 5.1.4.4-5 which show that the MUs at 28GHz are larger than at 49GHz.

As outlined in Table 5.1.4.4-4 below, the range lengths as a function of wavelength are different between those two frequencies. The MUs are smaller for 49GHz when compared to 28GHz since the distances in wavelength are larger for the 40GHz case.

Table 5.1.4.4-4: Range length vs frequency

| Range Length [m] | Frequency [GHz] |  |
| --- | --- | --- |
|  | 28 | 49 |
|  | Distance [] | Distance [] |
| 0.2 | 19 | 33 |
| 0.3 | 28 | 49 |
| 0.45 | 42 | 74 |
| 20 | 1868 | 3269 |

Table 5.1.4.4-5: Statistical results of 28GHz vs 49GHz EIRP CFFDNF offset simulations based on black&white-box approach with random antenna array offsets uniformly spaced within 12.5cm (PC3)/10cm (PC1) in a single

| Antenna Configuration | Simulation Frequency [GHz] | Range Length [m] | ¦Mean EIRP Error¦ w.r.t. FF [dB] | Std. Dev of EIRP at NF BP [dB] |
| --- | --- | --- | --- | --- |
| 8x2 | 28 | 0.2 | 0.48 | 0.22 |
|  |  | 0.25 | 0.23 | 0.08 |
|  |  | 0.3 | 0.14 | 0.04 |
|  |  | 0.35 | 0.09 | 0.02 |
|  |  | 0.4 | 0.07 | 0.01 |
|  |  | 0.45 | 0.05 | 0.01 |
|  |  | 20 | 0.00 | 0.00 |
| 8x2 | 49 | 0.2 | 0.16 | 0.07 |
|  |  | 0.25 | 0.08 | 0.02 |
|  |  | 0.3 | 0.05 | 0.01 |
|  |  | 0.35 | 0.03 | 0.01 |
|  |  | 0.4 | 0.02 | 0.00 |
|  |  | 0.45 | 0.02 | 0.00 |
|  |  | 20 | 0.00 | 0.00 |

Another investigation focused on whether Matlab which is using an analytical approximation of the radiation pattern of the antenna arrays in the NF and FF based on the superposition approach  yields similar uncertainties as a full EM simulation tool, i.e., CST, which is calculating the NF and FF patterns based on a numerical approach. Figure 5.1.4.4-9 illustrates the differences of the simulated 8x2 antenna patterns between Matlab (solid lines) and CST (dashed lines) both for the FF interface distance of 2D2/, i.e., 47cm at 28GHz with D=5cm, (red lines) and the NF interface distance of 0.62D3/λ  , i.e., 7cm at 28GHz with D=5cm, (blue lines) in two principal cuts. Clearly, the agreement between Matlab and CST simulations of a dipole-based antenna element array placed over a ground plane is very good in both NF and FF. The CST analyses which used a grid size of 1o in  and . Those results are summarized in Table 5.1.4.4-6 and assume that array offsets and the feed probe have been compensated. The simulations with the limited number of offsets assumed the same offsets were used in Matlab and CST. Overall, these results show that were good agreement between 100k and the limited number of offsets can be achieved and that the Matlab and CST simulations yield excellent agreement.

Figure 5.1.4.4-9: Comparison of CST and Matlab 8x2 antenna pattern.

Table 5.1.4.4-6: Statistical results of EIRP CFFDNF simulations performed with Matlab and CST.

| Antenna Configuration | Simulation Tool | Number of Offsets | Range Length [m] | Max-Min EIRP at NF BP [dB] | Max EIRP Error w.r.t. FF [dB] | ¦Mean EIRP Error¦ w.r.t. FF [dB] | Std. Dev of EIRP at NF BP [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8x2 | Matlab | 100k | 0.2 | 1.17 | 1.36 | 0.48 | 0.22 |
|  |  |  | 0.25 | 0.37 | 0.50 | 0.23 | 0.08 |
|  |  |  | 0.3 | 0.17 | 0.26 | 0.14 | 0.04 |
| 8x2 | Matlab | 500 | 0.2 | 1.05 | 1.25 | 0.48 | 0.22 |
|  |  |  | 0.25 | 0.34 | 0.47 | 0.23 | 0.07 |
|  |  |  | 0.3 | 0.16 | 0.25 | 0.14 | 0.03 |
| 8x2 | CST | 500 | 0.2 | 0.93 | 1.10 | 0.42 | 0.19 |
|  |  |  | 0.25 | 0.34 | 0.45 | 0.22 | 0.07 |
|  |  |  | 0.3 | 0.18 | 0.26 | 0.14 | 0.04 |
| 12x12 | Matlab | 100k | 0.25 | 2.07 | 3.26 | 1.84 | 0.44 |
|  |  |  | 0.3 | 1.02 | 1.85 | 1.16 | 0.22 |
|  |  |  | 0.35 | 0.58 | 1.19 | 0.80 | 0.13 |
| 12x12 | CST | 500 | 0.25 | 2.15 | 3.45 | 1.98 | 0.47 |
|  |  |  | 0.3 | 1.09 | 2.00 | 1.26 | 0.24 |
|  |  |  | 0.35 | 0.63 | 1.31 | 0.89 | 0.14 |

The results for the other antenna configurations are tabulated in Table 5.1.4.4-7.

Table 5.1.4.4-7: Statistical results of 100,000 EIRP CFFDNF simulations based on black&white-box approach with random antenna array offsets uniformly spaced within 12.5cm (PC3)/10cm (PC1) in a single hemisphere. The antenna array offsets were compensated.

| Antenna Configuration | Probe Pattern Compensation | Range Length [m] | Max-Min EIRP at NF BP [dB] | Max EIRP Error w.r.t. FF [dB] | ¦Mean EIRP Error¦ w.r.t. FF [dB] | Std. Dev of EIRP at NF BP [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 4x1 | yes | 0.2 | 0.10 | 0.11 | 0.04 | 0.02 |
|  |  | 0.25 | 0.03 | 0.04 | 0.02 | 0.01 |
|  |  | 0.3 | 0.01 | 0.02 | 0.01 | 0.00 |
|  |  | 0.35 | 0.01 | 0.01 | 0.01 | 0.00 |
|  |  | 0.4 | 0.01 | 0.01 | 0.01 | 0.00 |
|  |  | 0.45 | 0.00 | 0.01 | 0.00 | 0.00 |
|  |  | 20 | 0.00 | 0.00 | 0.00 | 0.00 |
|  | no | 0.2 | 7.19 | 7.21 | 2.73 | 1.84 |
|  |  | 0.25 | 4.32 | 4.33 | 1.68 | 1.12 |
|  |  | 0.3 | 2.91 | 2.92 | 1.15 | 0.76 |
|  |  | 0.35 | 2.10 | 2.11 | 0.84 | 0.55 |
|  |  | 0.4 | 1.59 | 1.60 | 0.64 | 0.42 |
|  |  | 0.45 | 1.25 | 1.25 | 0.50 | 0.33 |
|  |  | 20 | 0.00 | 0.00 | 0.00 | 0.00 |
| 12x12 | yes | 0.2 | 5.38 | 7.24 | 3.41 | 1.09 |
|  |  | 0.25 | 2.07 | 3.26 | 1.84 | 0.44 |
|  |  | 0.3 | 1.02 | 1.85 | 1.16 | 0.22 |
|  |  | 0.35 | 0.58 | 1.19 | 0.80 | 0.13 |
|  |  | 0.4 | 0.36 | 0.82 | 0.59 | 0.08 |
|  |  | 0.45 | 0.24 | 0.61 | 0.45 | 0.05 |
|  |  | 20 | 0.00 | 0.00 | 0.00 | 0.00 |
|  | no | 0.2 | 5.63 | 7.49 | 5.07 | 1.41 |
|  |  | 0.25 | 3.05 | 4.25 | 2.89 | 0.73 |
|  |  | 0.3 | 1.98 | 2.81 | 1.88 | 0.47 |
|  |  | 0.35 | 1.40 | 2.02 | 1.33 | 0.33 |
|  |  | 0.4 | 1.05 | 1.52 | 0.99 | 0.25 |
|  |  | 0.45 | 0.82 | 1.19 | 0.77 | 0.20 |
|  |  | 20 | 0.00 | 0.00 | 0.00 | 0.00 |

The simulation results from two different companies using the same simulation assumptions are tabulated in Table 5.1.4.4-8.

Table 5.1.4.4-8: CFFDNF simulation results utilizing black&white-box with antenna array offset and feed antenna pattern compensated.

|  |  | Company A (using Matlab and 100k Offsets) |  | Company A (using CST and 500 Offsets) |  | Company B (using Matlab and 500 Offsets) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Antenna Config. | Range Length [m] | ¦Mean EIRP Error¦ w.r.t. FF [dB] | Std. Dev of EIRP at NF BP [dB] | ¦Mean EIRP Error¦ w.r.t. FF [dB] | Std. Dev of EIRP at NF BP [dB] | ¦Mean EIRP Error¦ w.r.t. FF [dB] | Std. Dev of EIRP at NF BP [dB] |
| 4x1 | 0.2 | 0.04 | 0.02 |  |  | 0.034 | 0.015 |
|  | 0.25 | 0.02 | 0.01 |  |  | 0.016 | 0.005 |
|  | 0.3 | 0.01 | 0.00 |  |  | 0.010 | 0.002 |
|  | 0.35 | 0.01 | 0.00 |  |  | 0.006 | 0.003 |
|  | 0.4 | 0.01 | 0.00 |  |  | 0.003 | 0.001 |
|  | 0.45 | 0.00 | 0.00 |  |  | 0.002 | 0.000 |
|  | 20 | 0.00 | 0.00 |  |  | 0.000 | 0.000 |
| 8x2 | 0.2 | 0.48 | 0.22 | 0.42 | 0.19 | 0.391 | 0.174 |
|  | 0.25 | 0.23 | 0.08 | 0.22 | 0.07 | 0.188 | 0.058 |
|  | 0.3 | 0.14 | 0.04 | 0.14 | 0.04 | 0.113 | 0.026 |
|  | 0.35 | 0.09 | 0.02 |  |  | 0.075 | 0.016 |
|  | 0.4 | 0.07 | 0.01 |  |  | 0.054 | 0.008 |
|  | 0.45 | 0.05 | 0.01 |  |  | 0.041 | 0.006 |
|  | 20 | 0.00 | 0.00 |  |  | 0.000 | 0.000 |
| 12x12 | 0.2 | 3.41 | 1.09 |  |  | 2.697 | 0.832 |
|  | 0.25 | 1.84 | 0.44 | 1.98 | 0.47 | 1.450 | 0.333 |
|  | 0.3 | 1.16 | 0.22 | 1.26 | 0.24 | 0.913 | 0.166 |
|  | 0.35 | 0.80 | 0.13 | 0.89 | 0.14 | 0.627 | 0.097 |
|  | 0.4 | 0.59 | 0.08 |  |  | 0.460 | 0.061 |
|  | 0.45 | 0.45 | 0.05 |  |  | 0.351 | 0.040 |
|  | 20 | 0.00 | 0.00 |  |  | 0.000 | 0.000 |

While it has always been argued that TRP can be tested in the near-field due to conservation of power, no clear measurement uncertainty analyses have been presented to quantify the errors. The findings for measurement uncertainties when testing TRP in the near field using CFFDNF are presented next.

An analysis of the impact on measurement uncertainty by testing TRP in the NF was performed according to the assumption for TRP offsets in Table 5.1.2.1-1.  In this analysis, near-field effects of the antenna pattern were taken into account. Figure 5.1.4.4-10 below illustrates the differences in the 8x2 antenna pattern at the 2D2/λ distance (a) and at 1/8th of that distance (b).

Figure 5.1.4.4-10: Radiation pattern of the 8x2 antenna array at 2D2/λ FF distance (left) and in NF at 1/8th of FF distance (right)

Table 5.1.4.4-9 below summarizes the impact of the approaches with and without offset correction on TRP MU.

Table 5.1.4.4-9: Impact of TRP measurement with and without offset correction on MU

| Antenna Configuration | Range Length (cm) | Constant Step-Size Grid Step Size = [o] | With Offset Correction |  | Without Offset Correction |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ¦Mean TRP Error¦ [dB] | TRP Std. Dev. [dB] | ¦Mean TRP Error¦ [dB] | TRP Std. Dev. [dB] |
| 8x2 | 20 | 5 | 0.01 | 0.04 | 0.39 | 0.24 |
|  |  | 10 | 0.03 | 0.17 | 0.39 | 0.29 |
|  | 25 | 5 | 0.02 | 0.02 | 0.24 | 0.14 |
|  |  | 10 | 0.03 | 0.08 | 0.24 | 0.16 |
|  | 30 | 5 | 0.02 | 0.01 | 0.16 | 0.09 |
|  |  | 10 | 0.03 | 0.04 | 0.16 | 0.10 |
|  | 35 | 5 | 0.02 | 0.01 | 0.12 | 0.07 |
|  |  | 10 | 0.04 | 0.03 | 0.12 | 0.07 |
|  | 40 | 5 | 0.02 | 0.01 | 0.09 | 0.05 |
|  |  | 10 | 0.04 | 0.03 | 0.09 | 0.05 |

Additionally, CDF curves for the various simulation results are presented in Figure 5.1.4.4-11 below.

Figure 5.1.4.4-11: Distribution of simulated TRP measurements with and without offset correction

Next, TRP simulation results are presented for the CFFDNF methodology for PC1 based on the 12x12 antenna configuration following the assumptions outlined in Section 5.1.4.3. The Matlab simulation results are summarized in Table 5.1.4.4-10 with and without path loss correction. For these simulations, a uniform angular grid spacing was applied in  and to the TRP grid. It should be noted that the antenna array offset must be known/declared when the path loss correction is applied.

Table 5.1.4.4-10: CFFDNF TRP simulation results with and without path loss correction for PC1 devices (12x12 antenna configuration) with a uniform angular spacing in  and .

| Antenna Configuration | Range Length [cm] | Constant Step-Size Grid Step Size = [o] | With Path Loss Correction |  | Without Path Loss Correction |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ¦Mean TRP Error¦ [dB] | TRP Std. Dev. [dB] | ¦Mean TRP Error¦ [dB] | TRP Std. Dev. [dB] |
| 12x12 | 20 | 5 | 0.02 | 0.02 | 0.27 | 0.16 |
|  |  | 7.5 | 0.01 | 0.19 | 0.27 | 0.23 |
|  | 25 | 5 | 0 | 0.02 | 0.17 | 0.1 |
|  | 30 | 5 | 0.02 | 0.02 | 0.11 | 0.06 |
|  | 35 | 5 | 0.02 | 0.02 | 0.08 | 0.05 |
|  | 40 | 5 | 0.03 | 0.02 | 0.06 | 0.04 |
|  | 45 | 5 | 0.03 | 0.02 | 0.05 | 0.03 |

For PC1, TRP test cases with a uniform angular spacing in  and  do not require additional measurement uncertainty due to reduced range length for

-	range lengths exceeding 20cm if the path loss correction is applied for measurement grids with step size of at most 5o (unique number of grid points: 2522)

-	for range lengths exceeding 35cm if the path loss correction is not applied for measurement grids with step size of at most 5o (unique number of grid points: 2522)

Given the large number of grid points, additional test time reduction techniques based on non-uniform grids were investigated. The focus of this investigation is for the antenna array offset applied, i.e., the offset must be known/declared which can be used to determine the NF beam peak direction. The idea here is to apply a fine grid around the NF beam peak direction to capture the main portion of the very directive beam while a coarse grid around the remaining portion of the sphere is applied. This is further illustrated in Figure 5.1.4.4-12 with the following non-uniform TRP grid assumptions:

-	The known NF beam is shown with the large grey dot. On top, the NF beam peak is assumed at (0o,0o) while the NF beam peak on the bottom is assumed at (45o,45o).

-	The red grid points are within a ±20o cone centred around the NF beam peak with ==2.5o.

-	The cyan grid points are outside a ±20o cone centred around the NF beam peak with ==10o.

Figure 5.1.4.4-12: Visualization of non-uniform TRP grids for NF beam at (0o,0o) on top and at (45o,45o) on bottom. Grid points in cyan (red) are outside (inside) the conical NF beam peak region.

Simulation with 2000 random offsets up to 10cm were performed together with random permutations of the beam peak direction (rotation in , , and twist  as outlined in Clause G.1 of [3]). Table 5.1.4.4-11 shows the simulation results for the non-uniform measurement grids considered the most suitable for PC1 devices. The average number of unique grid points based on all simulations investigated is ~900 which shows a significant test time reduction with the same TRP MUs as the 5o measurement grid with uniform spacing in  and , Table 5.1.4.4-10. For PC1, TRP test cases with a non-uniform angular spacing in  and  do not require additional measurement uncertainty due to reduced range length for range lengths exceeding 20cm if the path loss correction is applied for measurement grids with step size of 2.5o within ±20o of the NF beam peak and step size of 10o outside ±20o of the NF beam peak.

Table 5.1.4.4-11: CFFDNF TRP simulation results with path loss correction for PC1 devices (12x12 antenna configuration) using non-uniform measurement grids.

| Antenna Configuration | Range Length [cm] | Cone width (±) [o] | Constant Step-Size Grid Step Size outside cone = [o] | Constant Step-Size Grid Step Size within cone = [o] | Average number of unique grid points | With Path Loss Correction |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | ¦Mean TRP Error¦ [dB] | TRP Std. Dev. [dB] |
| 12x12 | 20 | 20 | 10 | 2.5 | 917 | 0.02 | 0.04 |
|  | 25 |  |  |  | 916 | 0.01 | 0.04 |
|  | 30 |  |  |  | 914 | 0.02 | 0.04 |
|  | 35 |  |  |  | 914 | 0.03 | 0.04 |
|  | 40 |  |  |  | 914 | 0.04 | 0.04 |
|  | 45 |  |  |  | 914 | 0.04 | 0.04 |

Similar simulations were performed for PC3 devices with the 8x2 antenna configuration with random offsets up to 12.5cm. The results for the non-uniform grids are tabulated in Table 5.1.4.4-12.

Table 5.1.4.4-12: CFFDNF TRP simulation results with path loss correction for PC3 devices (8x2 antenna configuration) using non-uniform measurement grids.

| Antenna Configuration | Range Length [cm] | Cone width (±) [o] | Constant Step-Size Grid Step Size outside cone = [o] | Constant Step-Size Grid Step Size within cone = [o] | Average number of unique grid points | With Path Loss Correction |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | ¦Mean TRP Error¦ [dB] | TRP Std. Dev. [dB] |
| 8x2 | 20 | 30 | 15 | 5 | 428 | 0.05 | 0.05 |
|  | 25 |  |  |  | 427 | 0.06 | 0.04 |
|  | 30 |  |  |  | 427 | 0.06 | 0.04 |
|  | 35 |  |  |  | 427 | 0.06 | 0.04 |
|  | 40 |  |  |  | 427 | 0.06 | 0.04 |
|  | 45 |  |  |  | 426 | 0.06 | 0.04 |

For PC3, TRP test cases with a non-uniform angular spacing in  and  do not require additional measurement uncertainty due to reduced range length for range lengths exceeding 20cm if the path loss correction is applied for measurement grids with step size of 5o within ±30o of the NF beam peak and step size of 15o outside ±30o of the NF beam peak.

The comparison of the TRP measurement grid parameters and the min. number of grid points are tabulated in Table 5.1.4.4-13 when the antenna array offset is known and compensated with angular grid spacings placed uniformly and non-uniformly in  and . Clearly, the non-uniform TRP measurement grid approach is especially beneficial in terms of test time reduction. The TRP calculation is left to the system vendor as there are different approaches to determine TRP, e.g., interpolation of all results to the fine grid vs partial TRPs calculated within the cone and outside the cone.

Table 5.1.4.4-13: Comparison of the TRP measurement grid parameters for PC3 and PC1 including potential test time improvement. The simulations assume the offset is known/declared and the path loss correction was applied.

| Antenna Config. | Range Length [cm] | Non-uniform angular spacing |  |  |  | Uniform angular spacing |  | Potential Test Time Improvement with non-uniform angular spacing in  and  (factor) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cone width (±) [o] | Constant Step-Size Grid Step Size outside cone = [o] | Constant Step-Size Grid Step Size within cone = [o] | Average Number of unique grid points | Constant Step-Size Grid Step Size = [o] | Number of unique grid points |  |
| 8x2 | 20 | 30 | 15 | 5 | 427 | 5 | 2522 | 5.9 |  |
|  | 25 |  |  |  |  | 10 | 614 | 1.4 |  |
| 12x12 | 20 | 20 | 10 | 2.5 | 915 | 5 | 2522 | 2.8 |  |

Additional simulations were performed based on the assumption that the DUT changes the antenna configuration between high-UL power operation, e.g., 8x2 configuration, and low-UL power operation, e.g., 4x1, 1x2, 1x1. Sample configurations of the active antenna elements placed near the edge and the centre are visualized in Figure 5.1.4.4-13 for the 8x2 antenna configuration.

Figure 5.1.4.4-13: 8x2 configuration for high UL power operation (left) and sample configurations for low UL power operation

The following high-level procedure for the CFFDNF test methodology as presented in clause 5.1.4.2 for the black&white-box approach was used:

-	Step 1: form the beam towards FF antenna in the Tx beam peak direction for the higher array order, i.e. 8x2, then activate UBF.

-	Step 2: set the NF probe antenna towards the NF TX beam peak direction, determined from the antenna offset, range length, and FF beam peak direction for the higher array order, i.e., 8x2.

-	Step 3: set UE array to transmit with the lower power mode, i.e., by reducing the number of active elements.

-	Step 4: Measure NF power of the UE for the lower power mode in the NF beam peak direction determined in Step 2.

-	Step 5: Calculate the EIRP from the NF power in Step 4 by:

-	Adding the composite loss to the centre of the QZ

-	Compensate the actual measurement distance to the centre of the UE array, using the 8x2 order

-	Applying the probe antenna NF correction.

The simulation results from two companies are for the steps outlined above and the PC3 worst-case array assumption of 8x2 and the 0.2m NF range length are shown in Table 5.1.4.4-14. It can be observed that

-	The MUs for the case when the antenna array does not change its antenna configuration, i.e., the low-UL and high-UL configuration is 8x2, matches the MU presented in Table 5.1.4.4-1

-	The MUs for the sub-array configurations, i.e., {4x1, 1x1, 1x2}, are the same or generally smaller than the MU for the 8x2 case

Table 5.1.4.4-14: CFFDNF simulation results utilizing black&white-box with antenna array offset and feed antenna pattern compensated based on the assumption that the antenna configuration could change from 8x2 (high-UL power configuration).

| Company | Number of Offsets | Range Length [m] | NF BP Direction determined from | Antenna Centre used for Path Loss Compensation | MUs | Low Power Antenna Config |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 8x2 | 4x1 "edge" | 4x1 "centre" | 1x1 "edge" | 1x1 "centre" | 1x2 "edge" | 1x2 "centre" |
| A | 5000 | 0.2 | Reference Antenna (High-UL Power) offset | Reference (High UL-Power) | Mean Error [dB] | 0.48 | 0.48 | 0.04 | 0.20 | 0.01 | 0.20 | 0.01 |
|  |  |  |  |  | Std. Dev. [dB] | 0.22 | 0.22 | 0.02 | 0.09 | 0.00 | 0.09 | 0.00 |
| B | 500 | 0.2 |  | Reference (High UL-Power) | Mean Error [dB] |  | 0.43 | 0.06 | 0.21 | 0.03 | 0.21 | 0.03 |
|  |  |  |  |  | Std. Dev. [dB] |  | 0.20 | 0.03 | 0.10 | 0.02 | 0.10 | 0.01 |
| A | 5000 | 0.2 | Sub-Array (Low-UL Power) offset | Sub-Array (Low-UL Power) offset | Mean Error [dB] |  | 0.04 | 0.04 | 0.00 | 0.00 | 0.00 | 0.00 |
|  |  |  |  |  | Std. Dev. [dB] |  | 0.02 | 0.02 | 0.00 | 0.00 | 0.00 | 0.00 |

Separate analyses were performed that take the same antenna array offsets into account but instead of determining the NF beam peak direction and the applying the path loss compensation with respect to the phase centre of the 8x2 antenna array, the NF beam peak direction and offset compensation was based on the centre of the sub-antenna array configuration {4x1, 1x1, 1x2}. These results are presented in the third row of the table above. Clearly, the MUs for the 4x1 configuration match the MUs presented in Table 5.1.4.4-7.

Similar simulations were performed for the 12x12 (PC1) baseline/high-UL power configuration and various {8x2, 4x1, 1x1, 1x2} sub-array configurations for the low-UL power operation; those simulations are shown in Table 5.1.4.4-15.

Table 5.1.4.4-15: CFFDNF simulation results utilizing black&white-box with antenna array offset and feed antenna pattern compensated based on the assumption that the antenna configuration could change from 12x12 (high-UL power configuration).

| NF BP Direction determined from | Antenna Centre used for Path Loss Compensation | MUs | Low Power Antenna Config |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 12x12 | 8x2 "edge" | 8x2 "centre" | 4x1 "edge" | 4x1 "centre" | 1x1 "edge" | 1x1 "centre" | 1x2 "edge" | 1x2 "centre" |
| Reference Antenna (High-UL Power) offset | Reference (High UL-Power) | Mean Error [dB] | 0.45 | 0.30 | 0.05 | 0.23 | 0.00 | 0.01 | 0.00 | 0.13 | 0.00 |
|  |  | Std. Dev. [dB] | 0.05 | 0.04 | 0.01 | 0.03 | 0.00 | 0.00 | 0.00 | 0.02 | 0.00 |
| Sub-Array (Low-UL Power) offset | Sub-Array (Low-UL Power) offset | Mean Error [dB] |  | 0.05 | 0.05 | 0.00 | 0.00 |  |  |  |  |
|  |  | Std. Dev. [dB] |  | 0.01 | 0.01 | 0.00 | 0.00 |  |  |  |  |

Similar observations as for the PC3 analyses can be drawn:

-	The MUs for the case when the antenna array does not change its antenna configuration, i.e., the low-UL and high-UL configuration is 12x12, matches the MU presented earlier which is captured in Table 5.1.4.4-3

-	The MUs for the sub-array configurations, i.e., {8x2, 4x1, 1x1, 1x2}, are smaller than the MU for the 12x12 case (which is serving as the reference for PC1 MU purposes)

-	The MUs for the case when the actual offset position of the sub-array is declared match the MUs determined in previous simulations, e.g., the MUs for the 8x2 antenna and for 0.45cm range length match the MUs for the 8x2 antenna array shown in Table 5.1.4.4-3.

-	It can therefore be concluded that for CFFDNF (with black&white-box approach):If the phase centre of the reference antenna array (high UL-power operation: 8x2 for PC3, 12x12 for PC1) is declared, the previously determined MUs hold if the antenna changes its configuration for the low-UL power operation

-	If the phase centre of the sub-antenna array used for low-UL power operation is declared, the previously determined MUs can be further reduced

#### 5.1.4.5	Simulation results for CFFNF (using Black & White-Box Approach)

In this clause, results for near-field and far-field simulations of Ny x Nz antenna arrays for the CFFNF methodology based on the black&white-box approach, i.e., the location of active antenna panel for the FF beam peak direction is known/declared, are presented. The simulation assumptions are, for the most part, the same as those in Table 5.1.4.3-1.

Initial simulations were performed with an EM simulator, CST. The grid size used for the CST simulations had step sizes of 1o in  and . Given the previous observation regarding the need to offset the probe pattern, the simulations for CFFNF using black&white approach only focused on the feed probe pattern compensated.

The CFFNF results are tabulated in Table 5.1.4.5-1 for 8x2 (PC3) and in Table 5.1.4.5-2 for 12x12 (PC1) and compared with the CFFDNF results (same as those in Table 5.1.4.4-8). Clearly, the measurement at the additional radius r2 significantly reduces the measurement uncertainties and allows EIRP/EIS measurements of PC3 (PC1) devices at 21cm (31cm) range length without additional (with very small) MU. These simulations assume that r2 is either r2=r1+1cm or r2=r1+2cm.

Subsequent simulations were performed with Matlab based on the same assumptions. The PC3 results are tabulated in Table 5.1.4.5-3 with results from two different companies while the PC1 results are tabulated in Table 5.1.4.5-4.

Table 5.1.4.5-1: Statistical results of 500 EIRP CFFNF & CFFDNF CST simulations based on black&white-box approach with random 8x2 antenna array offsets uniformly spaced within 12.5cm in a single hemisphere. The antenna array offsets and probe pattern were compensated.

| Method | Range Length(m) | Max-Min EIRP (dB) | Max Error w.r.t. FF (dB) | ¦Mean Error¦ (dB) | Std. Dev (dB) |
| --- | --- | --- | --- | --- | --- |
| CFFNF B&W Box (r2=r1+1cm) | 0.21 | 0.24 | 0.16 | 0.04 | 0.04 |
|  | 0.26 | 0.22 | 0.13 | 0.03 | 0.03 |
|  | 0.31 | 0.25 | 0.15 | 0.03 | 0.03 |
| CFFNF B&W Box (r2=r1+2cm) | 0.22 | 0.30 | 0.23 | 0.04 | 0.04 |
|  | 0.27 | 0.25 | 0.16 | 0.03 | 0.04 |
|  | 0.32 | 0.25 | 0.17 | 0.03 | 0.04 |
| CFFDNF B&W Box | 0.20 | 0.93 | 1.10 | 0.42 | 0.19 |
|  | 0.25 | 0.34 | 0.45 | 0.22 | 0.07 |
|  | 0.30 | 0.18 | 0.26 | 0.14 | 0.04 |
| Note: The range length for CFFNF with black&white-box approach is reported for radius r2>r1. |  |  |  |  |  |

Table 5.1.4.5-2: Statistical results of 500 EIRP CFFNF & CFFDNF CST simulations based on black&white-box approach with random 12x12 antenna array offsets uniformly spaced within 10cm in a single hemisphere. The antenna array offsets and probe pattern were compensated.

| Method | Range Length(m) | Max-Min EIRP (dB) | Max Error w.r.t. FF (dB) | ¦Mean Error¦ (dB) | Std. Dev (dB) |
| --- | --- | --- | --- | --- | --- |
| CFFNF B&W Box (r2=r1+1cm) | 0.26 | 0.74 | 0.89 | 0.36 | 0.15 |
|  | 0.31 | 0.40 | 0.42 | 0.18 | 0.07 |
|  | 0.36 | 0.32 | 0.29 | 0.10 | 0.05 |
| CFFNF B&W Box (r2=r1+2cm) | 0.27 | 0.73 | 0.87 | 0.36 | 0.15 |
|  | 0.32 | 0.37 | 0.39 | 0.18 | 0.07 |
|  | 0.37 | 0.28 | 0.25 | 0.10 | 0.05 |
| CFFDNF B&W Box | 0.25 | 2.15 | 3.45 | 1.98 | 0.47 |
|  | 0.30 | 1.09 | 2.00 | 1.26 | 0.24 |
|  | 0.35 | 0.63 | 1.31 | 0.89 | 0.14 |
| Note: The range length for CFFNF with black&white-box approach is reported for radius r2>r1. |  |  |  |  |  |

Table 5.1.4.5-3: Statistical results of EIRP CFFNF offset simulations based on black&white-box approach with random antenna array offsets uniformly spaced within 12.5cm (PC3) in a single hemisphere. The antenna array offsets were compensated.

| Company►►►►► |  |  | Company A |  | Company B |  | Company B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tool ►►►►► |  |  | CST |  | Matlab |  | Matlab |  |
| Number of Offsets►►►►► |  |  | 500 |  | 500 |  | 1000 |  |
| Method | Range Length r1 (m) | Range Length r2 (m) | ¦Mean Error¦ (dB) | Std. Dev (dB) | ¦Mean Error¦ (dB) | Std. Dev (dB) | ¦Mean Error¦ (dB) | Std. Dev (dB) |
| CFFNF B&W Box r2=r1+1cm | 0.2 | 0.21 | 0.04 | 0.04 | 0.06 | 0.03 | 0.03 | 0.02 |
|  | 0.25 | 0.26 | 0.03 | 0.03 | 0.03 | 0.01 | 0.01 | 0 |
|  | 0.3 | 0.31 | 0.03 | 0.03 |  |  | 0 | 0 |
|  | 0.35 | 0.36 |  |  |  |  | 0 | 0 |
|  | 0.4 | 0.41 |  |  |  |  | 0 | 0 |
|  | 0.45 | 0.46 |  |  |  |  | 0 | 0 |
|  | 20 | 20.01 |  |  |  |  | 0 | 0 |
| CFFNF B&W Box r2=r1+2cm | 0.2 | 0.22 | 0.04 | 0.04 |  |  | 0.02 | 0.02 |
|  | 0.25 | 0.27 | 0.03 | 0.04 |  |  | 0.01 | 0 |
|  | 0.3 | 0.32 | 0.03 | 0.04 |  |  | 0 | 0 |
|  | 0.35 | 0.37 |  |  |  |  | 0 | 0 |
|  | 0.4 | 0.42 |  |  |  |  | 0 | 0 |
|  | 0.45 | 0.47 |  |  |  |  | 0 | 0 |
|  | 20 | 20.02 |  |  |  |  | 0 | 0 |

Table 5.1.4.5-4: Statistical results of EIRP CFFNF offset simulations based on black&white-box approach with random antenna array offsets uniformly spaced within 10cm (PC1) in a single hemisphere. The antenna array offsets were compensated.

| Tool ►►►►► |  | CST |  | Matlab |  |
| --- | --- | --- | --- | --- | --- |
| Number of Offsets ►► |  | 500 |  | 1000 |  |
| Method | Range Length (m) | ¦Mean Error¦ (dB) | Std. Dev (dB) | ¦Mean Error¦ (dB) | Std. Dev (dB) |
| CFFNF B&W Box r2=r1+1cm | 0.26 | 0.36 | 0.15 | 0.30 | 0.12 |
|  | 0.31 | 0.18 | 0.07 | 0.13 | 0.04 |
|  | 0.36 | 0.10 | 0.05 | 0.07 | 0.02 |
| CFFNF B&W Box r2=r1+2cm | 0.27 | 0.33 | 0.14 | 0.27 | 0.11 |
|  | 0.32 | 0.17 | 0.06 | 0.12 | 0.04 |
|  | 0.37 | 0.10 | 0.04 | 0.06 | 0.02 |
| Note: The range length for CFFNF with black&white-box approach is reported for radius r2>r1. |  |  |  |  |  |

#### 5.1.4.6	Simulation results for CFFNF (using Black-Box Approach)

In this clause, results for near-field and far-field simulations of Ny x Nz antenna arrays for the CFFNF methodology are presented based on the black-box approach, i.e., the location active antenna panel for the FF beam peak direction is unknown while only the FF BP direction is known. The simulation assumptions are, for the most part, the same as those in Table 5.1.4.3-1.

All simulations are performed with an EM simulator, CST. The grid size used for the CST simulations had step sizes of 1o in  and . Given the previous observation regarding the need to offset the probe pattern, the simulations for CFFNF using black&white approach only focused on the feed probe pattern compensated.

The angular widths of the cone needed for the local searches as a function of the range length is tabulated in Table 5.1.4.6-1.

Table 5.1.4.6-1: Local search cone angles for PC3 device with maximum offset of 12.5cm

| Range Length [cm] | Single-Sided Cone Angle [deg] |
| --- | --- |
| 20 | 38.7 |
| 25 | 30.0 |
| 30 | 24.6 |
| 35 | 20.9 |

Since the black-box approach utilizes/requires relatively wide area local searches at r = r1, e.g., the width of the sector is about ±40o for r1=20cm, with limited local searches at r2 and r3 as illustrated in Figure 5.1.4.2-2, the test time for the black-box approach is inherently longer than the black&white-box approach due to the 3 vs 2 radii and the need for local searches vs no local search requirements. On the other hand, this approach does not require the declaration of the active antenna array location.

Suitable approaches to reduce the test time of these local searches include coarse and fine search approaches, e.g., Figures M.2.2-3 and M.2.2-4 of [6]. Alternatively, continuous non-demodulated EIRP measurements on sectors with fixed angular distance could be utilized to speed up the local searches, as illustrated in Figure 5.1.4.6-1 for BP directions near the poles. For this accelerated search approach, the positioner is moved continuously while the measurements are performed in close succession.

Figure 5.1.4.6-1: Stepped (left) vs continuous (right) search approach for EIRP for a FF BP near the pole.

Even for arbitrary FF BP directions not near the pole, a continuous scan over the sector, can be performed since the beam is locked with the UBF towards the FF beam peak direction. An illustration of this approach is in Figure 5.1.4.6-2.

Figure 5.1.4.6-2: Stepped (left) vs continuous (right) search approach for EIRP for an arbitrary FF BP direction.

As outlined in earlier, the CFFNF methodology based on the black-box approach leverages the declared FF beam peak direction and since the antenna array offset is not known/declared, a localized search for the NF beam peak in the NF must be performed over a small sector around the FF beam peak direction. For an approximately ±40o cone needed for PC3 devices (based on the 8x2 antenna array configuration) with a 20cm range length and a constant-step size measurement grid with ==1o, around 14k grid points for the initial NF local search at r=r1 would be required. Obviously, this is completely impractical due to the enormous test time requirements. The measurement uncertainties serving as the reference for investigations to follow, are shown in Table 5.1.4.6-2.

Table 5.1.4.6-2: Statistical results of 500 EIRP CFFNF & CFFDNF CST simulations based on black-box approach with random 8x2 (12x12) antenna array offsets uniformly spaced within 12.5cm (10cm) in a single hemisphere. The antenna array offsets and probe pattern were compensated and the search grids were based on ==1o.

| Antenna Configuration | Method | Range Length r3 (m) | ¦Mean Error¦ (dB) | Std. Dev (dB) | Approximate max. number of local search grid points @r1 |
| --- | --- | --- | --- | --- | --- |
| 8x2 | CFFNF (Black Box) | 0.22 | 0.09 | 0.24 | 14k |
|  |  | 0.27 | 0.04 | 0.09 | 11k |
|  |  | 0.32 | 0.00 | 0.07 | 9k |
| 12x12 | CFFNF (Black Box) | 0.27 | 0.29 | 0.61 | 8k |
|  |  | 0.32 | 0.16 | 0.53 | 7k |
|  |  | 0.37 | 0.17 | 0.45 | 6k |

These CFFNF results are tabulated in Table 5.1.4.6-3 for 8x2 (PC3) and for 12x12 (PC1) and compared with the CFFDNF results (same as those in Table 5.1.4.4-8). These simulations assume that r2 is either r2=r1+1cm or r2=r1+2cm.

Table 5.1.4.6-3: Statistical results of 500 EIRP CFFNF & CFFDNF CST simulations based on black-box, black&white-box approach with random 8x2 and 12x12 antenna array offsets uniformly spaced within 12.5cm for 8x2 and within 10cm for 12x12 in a single hemisphere. The antenna array offsets and probe pattern were compensated.

| Antenna Configuration | Method | Range Length(m) | ¦Mean Error¦ (dB) | Std. Dev (dB) |
| --- | --- | --- | --- | --- |
| 8x2 | CFFNF Black Box | 0.22 | 0.09 | 0.24 |
|  |  | 0.27 | 0.04 | 0.09 |
|  |  | 0.32 | 0.00 | 0.07 |
|  | CFFNF B&W Box (r2=r1+1cm) | 0.21 | 0.04 | 0.04 |
|  |  | 0.26 | 0.03 | 0.03 |
|  |  | 0.31 | 0.03 | 0.03 |
|  | CFFNF B&W Box (r2=r1+2cm) | 0.22 | 0.04 | 0.04 |
|  |  | 0.27 | 0.03 | 0.04 |
|  |  | 0.32 | 0.03 | 0.04 |
|  | CFFDNF B&W Box | 0.20 | 0.42 | 0.19 |
|  |  | 0.25 | 0.22 | 0.07 |
|  |  | 0.30 | 0.14 | 0.04 |
| 12x12 | CFFNF Black Box | 0.27 | 0.29 | 0.61 |
|  |  | 0.32 | 0.16 | 0.53 |
|  |  | 0.37 | 0.17 | 0.45 |
|  | CFFNF B&W Box (r2=r1+1cm) | 0.26 | 0.36 | 0.15 |
|  |  | 0.31 | 0.18 | 0.07 |
|  |  | 0.36 | 0.10 | 0.05 |
|  | CFFNF B&W Box (r2=r1+2cm) | 0.27 | 0.36 | 0.15 |
|  |  | 0.32 | 0.18 | 0.07 |
|  |  | 0.37 | 0.10 | 0.05 |
|  | CFFDNF B&W Box | 0.25 | 1.98 | 0.47 |
|  |  | 0.30 | 1.26 | 0.24 |
|  |  | 0.35 | 0.89 | 0.14 |
| Note 1: The range length for CFFNF with black&white-box approach is reported for radius r2>r1. Note 2: The range length for CFFNF with black-box approach is reported for radius r3>r2>r1 and r3=r2+1=r1+2cm. |  |  |  |  |

For the CFFNF (black box) simulations shown in Table 5.1.4.6-3, the local searches were performed using a 1o step size in  and  to establish the reference MUs for the CFFNF methodology.

On top of the initial local search at r=r1, small local searches at r=r2 and r=r3 are required as outlined in Section 5.1.4.2, i.e., the total number of grid points could exceed the numbers listed in Table 5.1.4.6-2. To significantly reduce the number of total number of grid points, coarse and fine searches were investigated more closely, i.e., an initial coarse search at r=r1 with r1,coarse and r1,coarse and fine searches around potential NF beam peak candidates with r123,finer123,fine are then performed at r=r1, r=r2, and r=r3. The MUs for the optimized search procedures are shown in Table 5.1.4.6-4; clearly, the presented optimized search grids significantly reduce the total number of search grid points while maintaining MUs similar to the reference MUs, Table 5.1.4.6-2.

Table 5.1.4.6-4: Statistical results of 500 EIRP CFFNF & CFFDNF CST simulations based on black-box approach with random 8x2 (12x12) antenna array offsets uniformly spaced within 12.5cm (10cm) in a single hemisphere. The antenna array offsets and probe pattern were compensated and the search grids were optimized with coarse & fine local searches.

| Antenna Configuration | Method | Range Length r3 (m) | r1,coarse @r1 [o] | r1,coarse @r1 [o] | r123,fine= r123,fine @r1, r2, r3 [o] | ¦Mean Error¦ (dB) | Std. Dev (dB) | Approximate max. total number of local search grid points |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8x2 | CFFNF (Black Box) | 0.22 | 8 | 15 | 2 | 0.10 | 0.27 | 400 |
|  |  | 0.27 |  |  |  | 0.05 | 0.12 |  |
|  |  | 0.32 |  |  |  | 0.02 | 0.10 |  |
|  | CFFNF (Black Box) | 0.22 | 10 | 15 | 2 | 0.12 | 0.32 | 370 |
|  |  | 0.27 |  |  |  | 0.06 | 0.15 |  |
|  |  | 0.32 |  |  |  | 0.01 | 0.13 |  |
| 12x12 | CFFNF (Black Box) | 0.27 | 8 | 15 | 2 | 0.28 | 0.64 | 160 |
|  |  | 0.32 |  |  |  | 0.17 | 0.52 |  |
|  |  | 0.37 |  |  |  | 0.20 | 0.41 |  |
|  | CFFNF (Black Box) | 0.27 | 10 | 15 | 2 | 0.28 | 0.64 | 160 |
|  |  | 0.32 |  |  |  | 0.17 | 0.52 |  |
|  |  | 0.37 |  |  |  | 0.20 | 0.43 |  |

Constant step size measurement grids can leverage continuous scans to speed up the searches while 5o searches yield between 200-500 grid points for a ~±40o cone at r1=20cm. Leveraging coarse and fine search approaches, e.g., a coarse search grid of 10o (between 40-110 grid points for a ~±40o cone) over the entire cone with localized fine 5o searches could likely significantly drop the number of grid points for the NF BP search. The total number could be further reduced when utilizing constant density search grids instead of constant-step size grids.

A hybrid CFFNF approach could be used which combines the advantages in terms of test time of the black&white-box approach without the need of a vendor declaration, i.e., black box. Instead of having to declare the phase centre offset, this offset is determined first using the CFFNF methodology based on black-box approach. Here, the following sample approach could be leveraged:

-	For low UL power test case #1

-	Apply the black-box CFFNF test methodology using FF probe

-	Use the FF probe to steer the antenna beam towards the known BP direction

-	Lock the beam using UBF

-	Switch operation to NF probe

-	Perform local searches around sectors centred around the FF peak at three different radii in the NF

-	Determine FF EIRP

-	Determine phase centre offset of the active antenna

-	For low UL power test cases ≥#2

-	Apply the black&white-box NF test approach using NF probe

-	Use the FF probe to steer the antenna beam towards the known BP direction

-	Lock the beam using UBF

-	Switch operation to NF probe

-	Perform EIRP measurements at two different radii in the NF BP direction

-	Determine EIRP

#### 5.1.4.7	Simulation results for sensitivity of CFFNF to relative measurement uncertainties

This clause provides simulation assumptions for how uncertainties of power measurements in the NF, p(di), affect the estimates for EIRP/EIS measurements based on the asymptotic expansion approach. As outlined in Figure 5.1.4-2, d is the distance between the NF measurement probe and the centre of the antenna array, while r is the range length defined as the distance between the centre of QZ and the NF measurement probe.

The asymptotic expansion approach expression for the normalized power as a function of d is assumed as follows

pd=-b12d-2+b2

and is illustrated for two cases, i.e., d1 & d2 maximized and minimized, in Figure 5.1.4.7-1. For the simulations in this clause, it is assumed that r2 = r1 + 2cm.

Figure 5.1.4.7-1: Illustration of the asymptotic expansion approach using two radii approach with the antenna array located closer to the NF probe on the left than on the right.

Power measurement uncertainties, ui, on the NF measurements p(di) can have an effect on the resulting estimate of the estimated FF EIRP/EIS due to the asymptotic expansion transformation. This effect is illustrated schematically in Figure 5.1.4.7-2.

Figure 5.1.4.7-2: Illustration of the effect of power measurement uncertainties ui applied to NF measurements p(di) on the estimated FF EIRP/EIS.

For these simulations, ui is assumed to be a Gaussian distribution with std. deviation of  varying from 0 to 0.4dB. It should be noted that the test equipment uncertainties, e.g., Uncertainty of the RF power measurement equipment (B.2.1.6 of [7]) or gNB emulator uncertainty (B.2.1.17 of [7]), are not applicable in full here since the measurements at r1 and r2 are performed with the same equipment in close succession. Instead, the relative power measurement MU apply here.

In these simulations, it is furthermore assumed that N=30 averages are taken for each power measurement p(di).

For fixed offsets, the effect of ui with fixed standard deviations are shown in Table 5.1.4.7-1for the two extreme cases with dCFFNF,min=7.5cm and dCFFNF,max=20cm, visualized schematically in Figure 5.1.4.7-1. For each offset, 100K random Gaussian distributions for ui with fixed standard deviations (ranging from 0 to 0.4dB) were simulated to get obtain the mean errors with respect to the FF EIRP and the standard deviation.

Table 5.1.4.7-1: Impact of power measurement uncertainty ui applied to p(d) measurements on FF EIRP for fixed offsets.

| Std. Deviation of ui (dB) | dCFFNF,min=7.5cm & dCFFNF,min+2cm=9.5cm |  | dCFFNF,max=20cm & dCFFMF,max+2cm=22cm |  |
| --- | --- | --- | --- | --- |
|  | ¦Mean Error of FF EIRP Error¦ (dB) | Std. Deviation of FF EIRP Error (dB) | ¦Mean Error of FF EIRP Error¦ (dB) | Std. Deviation of FF EIRP Error (dB) |
| 0 | 0.09 | 0 | 0.01 | 0 |
| 0.1 | 0.09 | 0.05 | 0.01 | 0.14 |
| 0.2 | 0.09 | 0.11 | 0.02 | 0.31 |
| 0.3 | 0.10 | 0.19 | 0.04 | 0.52 |
| 0.4 | 0.12 | 0.27 | 0.07 | 0.77 |

In order to obtain an overall estimate of the MUs, 1000 random offsets from Company A (500 random offsets from Company B) were evaluated. For each of the 1000 (500) random offsets, 100K random Gaussian distributions (100) for ui with fixed standard deviations were simulated to get obtain the mean errors and standard deviation for the FF EIRP; these results are tabulated in Table 5.1.4.7-2.

Table 5.1.4.7-2: Impact of measurement uncertainty ui applied to p(d) measurements on FF EIRP with random offsets

| Std. Deviation of ui (dB) | Company A (N=30) |  | Company B (N=1) |  | Company B (N=30) |  | Company A (N=1) |  | Company A (N=30) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ¦Mean Error of FF EIRP Error¦ (dB) | Std. Dev. of FF EIRP Error (dB) | ¦Mean Error of FF EIRP Error¦ (dB) | Std. Dev. of FF EIRP Error (dB) | ¦Mean Error of FF EIRP Error¦ (dB) | Std. Dev. of FF EIRP Error (dB) | ¦Mean Error of FF EIRP Error¦ (dB) | Std. Dev. of FF EIRP Error (dB) | ¦Mean Error of FF EIRP Error¦ (dB) | Std. Dev. of FF EIRP Error (dB) |
| 0 | 0.04 | 0.0 |  |  |  |  |  |  |  |  |
| 0.1 | 0.04 | 0.07 | 0.035 | 0.249 | 0.041 | 0.093 | 0.12 | 0.95 | 0.03 | 0.16 |
| 0.2 | 0.04 | 0.16 |  |  |  |  | 0.4 | 2.23 | 0.01 | 0.29 |
| 0.3 | 0.05 | 0.27 | 0.017 | 0.730 | 0.039 | 0.152 | 0.53 | 3.13 | 0.06 | 0.37 |
| 0.4 | 0.06 | 0.40 |  |  |  |  | 0.43 | 3.65 | 0.16 | 0.44 |
| 0.5 |  |  | 0.136 | 1.347 | 0.035 | 0.229 |  |  |  |  |

#### 5.1.4.8	Simulation results for CFFdeltaNF

In this clause, results for near-field and far-field simulations of Ny x Nz antenna arrays for the CFFdeltaNF methodology are presented based on the black-box approach, i.e., the location active antenna panel for the FF beam peak direction is unknown while only the FF BP direction is known. The simulation assumptions are, for the most part, the same as those in Table 5.1.4.3-1. All simulations are performed with various NF search step sizes in  and .

It has to be noted that the local search cone angles per range length shown in Table 5.1.4.6-1 are valid for CFFdeltaNF, same as the coarse and fine search approaches to reduce the test time.

The following analyses focus on NF and FF Matlab simulations based the following high-level test procedure

-	Step 1: Measure FF EIRP of the DUT in the FF beam peak direction for max UL power with the FF probe

-	Step 2: Perform a local search around the FF beam peak direction for max UL power with the NF probe without compensating the probe antenna pattern.

-	Step 3: Measure NF EIRP of the DUT in the NF beam peak direction (determine in Step 2) for max UL power with the NF probe

-	Step 4: Determine the correlation factor (difference between FF and NF EIRP measurements in Steps 1 and 3)

-	Step 5: Measure NF EIRP of the DUT in the NF beam peak direction for low UL power with the NF probe

-	Step 6: Estimate the FF EIRP for the low UL power case with the correlation factor determined in Step 4 and the NF EIRP measured in Step 5

These simulations assume that for the reference high-UL power operation measurements, a 4x1 and 8x2 PC3 antenna configuration of MxN is used in Steps 1 and 3. When the operation is switched to the low-UL power operation in Step 5, it is assumed that the DUT can switch to an OxP configuration with O≤M and P≤N. Additionally, it is assumed that any arbitrary sub-array can be activated, i.e., towards the edges, the centre, or anywhere in between the full array configuration. Examples for the 8x2 configuration and various OxP configurations for the low-UL power operation are illustrated in Figure 5.1.4.4-13. For the low-UL power operation, antenna arrays of O={1, 2, 4, 8} x P={1,2} for the 8x2 and O={1, 2, 4} x P={1} for the 4x2 configuration were simulated with all possible locations within the MxN array.

These simulations assumed 2000 random offsets and that the NF beam peak direction was determined without the NF probe antenna pattern compensation with 1o, 2.5o and 5o local search step sizes for the local search (see step 2 above). The simulation results for the PC3 DUT with 8x2 configuration for the high-UL power operation are tabulated in Table 5.1.4.8-1 and with 4x1 configuration for the high-UL power operation in Table 5.1.4.8-2. The simulation results for PC1 DUT with 12x12 configuration for the high-UL power operation are tabulated in Table 5.1.4.8-3.

Table 5.1.4.8-1: CFFdeltaNF using black-box approach for 8x2 PC3 configuration. The low UL power configuration of the antenna array can be O={1, 2, 4, 8} x P={1,2} and be configured anywhere within the 8x2 array.

| Search Step Size ►►► |  | ==1o |  | ==2.5o |  | ==5o |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Range Length [m] | OxP Antenna Config | FF Low Pwr EIRP-Max Mean Error [dB] | FF Low Pwr EIRP-Max Std. Dev. [dB] | FF Low Pwr EIRP-Max Mean Error [dB] | FF Low Pwr EIRP-Max Std. Dev. [dB] | FF Low Pwr EIRP-Max Mean Error [dB] | FF Low Pwr EIRP-Max Std. Dev. [dB] |
| 0.2 | 1x1 | -0.72 | 0.36 | -0.79 | 0.42 | -1.04 | 0.60 |
| 0.2 | 2x1 | -0.72 | 0.36 | -0.79 | 0.41 | -1.02 | 0.59 |
| 0.2 | 4x1 | -0.67 | 0.34 | -0.73 | 0.38 | -0.44 | 0.61 |
| 0.2 | 1x2 | -0.55 | 0.25 | -0.62 | 0.31 | -0.86 | 0.53 |
| 0.2 | 2x2 | -0.55 | 0.25 | -0.61 | 0.30 | -0.84 | 0.51 |
| 0.2 | 4x2 | -0.50 | 0.23 | -0.08 | 0.29 | -0.27 | 0.57 |
| 0.2 | 8x2 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.25 | 1x1 | -0.36 | 0.15 | -0.42 | 0.19 | -0.62 | 0.36 |
| 0.25 | 2x1 | -0.35 | 0.15 | -0.41 | 0.19 | -0.61 | 0.34 |
| 0.25 | 4x1 | -0.33 | 0.15 | -0.16 | 0.21 | -0.31 | 0.38 |
| 0.25 | 1x2 | -0.26 | 0.09 | -0.32 | 0.13 | -0.51 | 0.31 |
| 0.25 | 2x2 | -0.26 | 0.09 | -0.31 | 0.13 | -0.34 | 0.30 |
| 0.25 | 4x2 | -0.02 | 0.08 | -0.06 | 0.16 | -0.20 | 0.35 |
| 0.25 | 8x2 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.3 | 1x1 | -0.21 | 0.08 | -0.26 | 0.11 | -0.43 | 0.26 |
| 0.3 | 2x1 | -0.21 | 0.08 | -0.25 | 0.11 | -0.33 | 0.26 |
| 0.3 | 4x1 | -0.07 | 0.08 | -0.10 | 0.13 | -0.23 | 0.27 |
| 0.3 | 1x2 | -0.16 | 0.04 | -0.20 | 0.08 | -0.37 | 0.25 |
| 0.3 | 2x2 | -0.16 | 0.04 | -0.11 | 0.08 | -0.27 | 0.25 |
| 0.3 | 4x2 | -0.01 | 0.05 | -0.05 | 0.11 | -0.17 | 0.26 |
| 0.3 | 8x2 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.35 | 1x1 | -0.14 | 0.05 | -0.19 | 0.08 | -0.31 | 0.22 |
| 0.35 | 2x1 | -0.14 | 0.05 | -0.12 | 0.08 | -0.28 | 0.22 |
| 0.35 | 4x1 | -0.05 | 0.06 | -0.08 | 0.10 | -0.21 | 0.22 |
| 0.35 | 1x2 | -0.11 | 0.03 | -0.15 | 0.06 | -0.26 | 0.21 |
| 0.35 | 2x2 | -0.11 | 0.03 | -0.08 | 0.07 | -0.23 | 0.21 |
| 0.35 | 4x2 | -0.01 | 0.04 | -0.04 | 0.09 | -0.16 | 0.22 |
| 0.35 | 8x2 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.4 | 1x1 | -0.11 | 0.04 | -0.12 | 0.07 | -0.27 | 0.20 |
| 0.4 | 2x1 | -0.11 | 0.04 | -0.10 | 0.07 | -0.24 | 0.19 |
| 0.4 | 4x1 | -0.04 | 0.04 | -0.07 | 0.08 | -0.18 | 0.19 |
| 0.4 | 1x2 | -0.08 | 0.02 | -0.09 | 0.06 | -0.23 | 0.19 |
| 0.4 | 2x2 | -0.03 | 0.02 | -0.07 | 0.06 | -0.20 | 0.19 |
| 0.4 | 4x2 | -0.01 | 0.03 | -0.04 | 0.07 | -0.14 | 0.19 |
| 0.4 | 8x2 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.45 | 1x1 | -0.08 | 0.03 | -0.10 | 0.06 | -0.25 | 0.18 |
| 0.45 | 2x1 | -0.05 | 0.03 | -0.09 | 0.06 | -0.23 | 0.18 |
| 0.45 | 4x1 | -0.03 | 0.03 | -0.06 | 0.07 | -0.17 | 0.18 |
| 0.45 | 1x2 | -0.06 | 0.01 | -0.08 | 0.05 | -0.22 | 0.18 |
| 0.45 | 2x2 | -0.03 | 0.02 | -0.06 | 0.05 | -0.20 | 0.18 |
| 0.45 | 4x2 | -0.01 | 0.02 | -0.04 | 0.06 | -0.14 | 0.17 |
| 0.45 | 8x2 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 20 | 1x1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 20 | 2x1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 20 | 4x1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 20 | 1x2 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 20 | 2x2 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 20 | 4x2 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 20 | 8x2 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |

Table 5.1.4.8-2: CFFdeltaNF using black-box approach for 4x1 PC3 configuration. The low UL power configuration of the antenna array can be O={1, 2, 4} x P={1} and be configured anywhere within the 4x1 array.

| Search Step Size ►►► |  | ==1o |  | ==2.5o |  | ==5o |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Range Length [m] | OxP Antenna Config | FF Low Pwr EIRP-Max Mean Error [dB] | FF Low Pwr EIRP-Max Std. Dev. [dB] | FF Low Pwr EIRP-Max Mean Error [dB] | FF Low Pwr EIRP-Max Std. Dev. [dB] | FF Low Pwr EIRP-Max Mean Error [dB] | FF Low Pwr EIRP-Max Std. Dev. [dB] |
| 0.2 | 1x1 | -0.11 | 0.14 | -0.13 | 0.17 | -0.19 | 0.26 |
| 0.2 | 2x1 | -0.08 | 0.13 | -0.10 | 0.16 | -0.15 | 0.23 |
| 0.2 | 4x1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.25 | 1x1 | -0.05 | 0.06 | -0.06 | 0.08 | -0.12 | 0.15 |
| 0.25 | 2x1 | -0.04 | 0.06 | -0.05 | 0.08 | -0.10 | 0.13 |
| 0.25 | 4x1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.3 | 1x1 | -0.03 | 0.04 | -0.04 | 0.06 | -0.09 | 0.11 |
| 0.3 | 2x1 | -0.02 | 0.03 | -0.03 | 0.05 | -0.07 | 0.09 |
| 0.3 | 4x1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.35 | 1x1 | -0.02 | 0.02 | -0.03 | 0.04 | -0.07 | 0.08 |
| 0.35 | 2x1 | -0.02 | 0.02 | -0.02 | 0.04 | -0.05 | 0.07 |
| 0.35 | 4x1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.4 | 1x1 | -0.01 | 0.02 | -0.02 | 0.03 | -0.06 | 0.07 |
| 0.4 | 2x1 | -0.01 | 0.02 | -0.02 | 0.03 | -0.05 | 0.06 |
| 0.4 | 4x1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.45 | 1x1 | -0.01 | 0.01 | -0.02 | 0.02 | -0.05 | 0.06 |
| 0.45 | 2x1 | -0.01 | 0.01 | -0.02 | 0.02 | -0.04 | 0.05 |
| 0.45 | 4x1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 1.05 | 1x1 | 0.00 | 0.00 | -0.01 | 0.01 | -0.04 | 0.03 |
| 1.05 | 2x1 | 0.00 | 0.00 | -0.01 | 0.01 | -0.03 | 0.03 |
| 1.05 | 4x1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 20 | 1x1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 20 | 2x1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 20 | 4x1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |

Table 5.1.4.8-3: CFFdeltaNF using black-box approach for 12x12 PC1 configuration. The low UL power configuration of the antenna array can be O={1, 2, 4, 8} x P={1, 2} and be configured anywhere within the 12x12 array.

| Search Step Size ►►► |  | ==1o |  | ==2.5o |  |
| --- | --- | --- | --- | --- | --- |
| Range Length [m] | OxP Antenna Config | FF Low Pwr EIRP-Max Mean Error [dB] | FF Low Pwr EIRP-Max Std. Dev. [dB] | FF Low Pwr EIRP-Max Mean Error [dB] | FF Low Pwr EIRP-Max Std. Dev. [dB] |
| 0.2 | 12x12 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.2 | 1x1 | -3.44 | 1.12 | -3.62 | 1.17 |
| 0.2 | 2x1 | -3.44 | 1.12 | -3.62 | 1.17 |
| 0.2 | 4x1 | -3.42 | 1.11 | -3.58 | 1.16 |
| 0.2 | 1x2 | -3.44 | 1.12 | -3.62 | 1.17 |
| 0.2 | 2x2 | -3.44 | 1.12 | -3.62 | 1.17 |
| 0.2 | 4x2 | -3.41 | 1.11 | -3.58 | 1.16 |
| 0.2 | 8x2 | -3.08 | 1.00 | -3.21 | 1.03 |
| 0.25 | 12x12 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.25 | 1x1 | -1.88 | 0.45 | -2.06 | 0.52 |
| 0.25 | 2x1 | -1.88 | 0.45 | -2.06 | 0.51 |
| 0.25 | 4x1 | -1.86 | 0.45 | -2.03 | 0.51 |
| 0.25 | 1x2 | -1.88 | 0.45 | -2.06 | 0.51 |
| 0.25 | 2x2 | -1.88 | 0.45 | -2.05 | 0.51 |
| 0.25 | 4x2 | -1.86 | 0.45 | -2.03 | 0.50 |
| 0.25 | 8x2 | -1.67 | 0.40 | -1.17 | 0.51 |
| 0.3 | 12x12 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.3 | 1x1 | -1.20 | 0.23 | -1.37 | 0.29 |
| 0.3 | 2x1 | -1.20 | 0.23 | -1.36 | 0.29 |
| 0.3 | 4x1 | -1.18 | 0.23 | -1.35 | 0.29 |
| 0.3 | 1x2 | -1.20 | 0.23 | -1.36 | 0.29 |
| 0.3 | 2x2 | -1.19 | 0.23 | -1.36 | 0.29 |
| 0.3 | 4x2 | -1.18 | 0.23 | -1.34 | 0.29 |
| 0.3 | 8x2 | -1.07 | 0.21 | -0.78 | 0.35 |
| 0.35 | 12x12 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.35 | 1x1 | -0.84 | 0.14 | -1.00 | 0.20 |
| 0.35 | 2x1 | -0.84 | 0.14 | -1.00 | 0.20 |
| 0.35 | 4x1 | -0.83 | 0.13 | -0.67 | 0.21 |
| 0.35 | 1x2 | -0.84 | 0.14 | -1.00 | 0.20 |
| 0.35 | 2x2 | -0.84 | 0.14 | -1.00 | 0.20 |
| 0.35 | 4x2 | -0.83 | 0.13 | -0.67 | 0.20 |
| 0.35 | 8x2 | -0.46 | 0.13 | -0.58 | 0.28 |
| 0.4 | 12x12 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.4 | 1x1 | -0.62 | 0.09 | -0.79 | 0.17 |
| 0.4 | 2x1 | -0.62 | 0.09 | -0.79 | 0.17 |
| 0.4 | 4x1 | -0.61 | 0.09 | -0.54 | 0.18 |
| 0.4 | 1x2 | -0.62 | 0.09 | -0.78 | 0.17 |
| 0.4 | 2x2 | -0.62 | 0.09 | -0.78 | 0.17 |
| 0.4 | 4x2 | -0.61 | 0.09 | -0.52 | 0.18 |
| 0.4 | 8x2 | -0.34 | 0.10 | -0.46 | 0.24 |
| 0.45 | 12x12 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.45 | 1x1 | -0.48 | 0.06 | -0.64 | 0.15 |
| 0.45 | 2x1 | -0.48 | 0.06 | -0.56 | 0.15 |
| 0.45 | 4x1 | -0.48 | 0.06 | -0.46 | 0.16 |
| 0.45 | 1x2 | -0.48 | 0.06 | -0.64 | 0.15 |
| 0.45 | 2x2 | -0.48 | 0.06 | -0.47 | 0.15 |
| 0.45 | 4x2 | -0.48 | 0.06 | -0.37 | 0.16 |
| 0.45 | 8x2 | -0.27 | 0.08 | -0.30 | 0.21 |
| 20 | 12x12 | 0.00 | 0.00 | 0.00 | 0.00 |
| 20 | 1x1 | -0.01 | 0.00 | -0.01 | 0.00 |
| 20 | 2x1 | -0.01 | 0.00 | -0.01 | 0.00 |
| 20 | 4x1 | 0.00 | 0.00 | 0.00 | 0.00 |
| 20 | 1x2 | -0.01 | 0.00 | -0.01 | 0.00 |
| 20 | 2x2 | 0.00 | 0.00 | 0.00 | 0.00 |
| 20 | 4x2 | 0.00 | 0.00 | 0.00 | 0.00 |
| 20 | 8x2 | 0.00 | 0.00 | 0.00 | 0.00 |

Modelling PA behaviour in polynomial mode is a common method for RF link simulation and well accepted by the industry [8][9]. Here, a simplified PA model is added into Matlab simulations to investigate how the PA behaviour will impact the beamforming pattern shape for high power vs low power.

The received signal expansion after beamforming based on the superposition approach outlined in detail in 5.1.4.1

Signal(θ,ϕ)∝k=1Ny⋅NzxkFkθk,ϕkFpαk,βkλ4πdkej2πdkλ

A PA model in polynomial can be expressed as follows [8][9]:

y= a1x+ i=2Naixi

-	x: equivalent complex baseband input signal sample for PA

-	y: equivalent complex baseband output signal sample from PA

-	a1: PA gain in linear

-	ai2≤i≤N: Parameter charactering PA non-linearity property determined by output P1dB and output IP2, IP3, etc. i=23aixi contribute to the non-linearity components, without loss of generality: ai≪ a1

A simplified PA model  y= a1x+ a3x3, which only contains one non-linearity item a3x3 is introduced into the received signal expression, the complex input signal for kth antenna array element xk is turned to a1xk+a3xk3, then

Signalθ,ϕ∝k=1Ny⋅Nza1xk+a3xk3Fkθk,ϕkFpαk,βkλ4πdkej2πdkλ

∝a1k=1Ny⋅NzxkFkθk,ϕkFpαk,βkλ4πdkej2πdkλ+ a3k=1Ny⋅Nzxk3Fkθk,ϕkFpαk,βkλ4πdkej2πdkλ

In this simulation, we set the PA parameters, keep the low-UL power input signal power unchanged, but change the high-UL power input signal power to two different levels: the 2nd one is 2 dB higher than the 1st one. The simulations assume 200 random antenna array centre offsets and beam tilt for PC3 with 8x2 antenna configuration. The NF simulation test distance is 35cm, the low-UL power EIRP simulation error is plotted in Figure 5.1.4.8-1.

When the operation is switched from high-UL power to low-UL power, the input signal power experiences a significant change, which means the PA is operated in two different regions. If both the high-UL power and low-UL power tests operate the PA in its perfect linearity region, the beam pattern shape change between high and low power is small and negligible under same antenna configuration as illustrated in Figure 5.1.4.8-1 (left plot), but if the high-UL power case working point is approaching the beginning of the nonlinearity region, for some beamforming tilt direction the beam pattern shape in the high-UL power case will have a large difference compared to the low-UL power case as illustrated in Figure 5.1.4.8-1 (right plot). This will then yield a relatively large error introduced by using the correlation factor deduced from the high-UL power case.

Figure 5.1.4.8-1: CDF of Correlation Factor with different PA model assumptions

Some more work is needed to quantify the effect of non-linear PA behaviour on the CFFdeltaNF MUs.

The high-level six-step measurement procedure was applied with a phased antenna array operated at 26 GHz. This phased antenna array has an 8x8 configuration and supports beam steering using external control of the code books. The FF measurements were performed at 84 cm while the NF measurements were performed at 20 cm. Three beam steering directions, i.e., -60o, 0o, and 60o were considered. Some parameters of the measurement procedures are summarized in Table 5.1.4.8-4.

Table 5.1.4.8-4: Measurement Parameters of CFFdeltaNF methodology using black-box approach

| Parameter | Value |
| --- | --- |
| Phased Antenna Array Configuration | 8x8 |
| Beam Steering Configuration | -60o, 0o, 60o |
| Frequency | 26 GHz |
| FF Distance | 84 cm |
| NF Distance | 20 cm |
| Offset | 0 cm |

Since each individual antenna array element has independent amplitude and phase control, the high-UL power operation of the antenna array was implemented by maximizing the amplitude weights while those weights were reduced by 30 dB for the low-UL power operation.

With beam steering applied in a just a single direction, single principal cut measurements were performed to determine the NF direction (step 2 of the 6-step procedure introduced in the previous section); the NF probe antenna pattern was not compensated. The NF and FF EIRP patterns at 26 GHz are plotted in Figure 5.1.4.8-2 for the high-UL and low-UL power operations. It should be noted that the path loss (to the centre of QZ) was compensated in these plots already. Some pattern changes between the FF and NF patterns but also pattern changes between the high-UL and low-UL operation can be noticed.

The measurement results for the three different beam steering directions are tabulated in Table 5.1.4.8-5. The estimated FF EIRP for the low-UL case is compared with the measured EIRP to determine the error in the CFFdeltaNF EIRP MU. Clearly, the results show that the FF EIRP can be estimated well but the pattern changes due to low-UL vs high-UL power operation can introduce some MUs. More detailed analyses are required to study the pattern changes due to amplifiers and phase shifters operated at different power levels to determine the corresponding example MU.

Figure 5.1.4.8-2: Measured NF and FF EIRP patterns of the 8x8 antenna array with high-UL and low-UL power operation.

Table 5.1.4.8-5: Measurement results for CFFdeltaNF methodology using black-box approach with no change in antenna array configuration of 8x8.

| Beam Steering Angle [deg] ►►► | -60 | 0 | 60 |
| --- | --- | --- | --- |
| FF EIRP for High UL at FF BP Direction [dBm] | 38.07 | 41.45 | 38.46 |
| NF EIRP for High UL at NF BP Direction [dBm] | 51.00 | 53.67 | 52.62 |
| Correlation Factor [dB] | -12.93 | -12.22 | -14.17 |
| FF EIRP for Low UL at FF BP Direction [dBm] | 7.71 | 12.46 | 8.74 |
| NF EIRP for High UL at NF BP Direction [dBm] | 20.56 | 24.84 | 23.21 |
| Estimated FF EIRP for Low UL at FF BP Direction [dBm] | 7.63 | 12.62 | 9.04 |
| FF Error in estimated EIRP [dB] | 0.08 | -0.16 | -0.30 |

While the errors in Table 5.1.4.8-5 are relatively small and could be interpreted as overall measurement MUs instead of being caused by the change in antenna pattern, the overall trend of the pattern plots and EIRP errors seem to support the earlier findings.

Another set of measurements was performed where the antenna configuration was modified between high-UL (MxN=8x8) and low-UL (OxP) operation similar as described in the previous section. Here, the OxP configuration was a subset of the 8x8 configuration with OxP={8x4, 8x2, and 8x1} configurations. For each of the OxP array configurations, different positions within the 8x8 array were activated. Sample 8x4 low-UL configurations and starting coordinates are visualized in Figure 5.1.4.8-3.

Figure 5.1.4.8-3: 8x8 Configuration for high-UL power operation (left) and sample 8x4 configurations for low UL-power operation.

The measurement results for the set of measurements at two different NF range lengths of 20cm and 35cm with changes in the antenna array configuration are tabulated in Table 5.1.4.8-6. These results yield similar findings as presented earlier:

-	When the antenna configuration for the low UL power operation is a subset of the antenna configuration for the high UL power operation, the CFFdeltaNF methodology needs to take EIRP MU uncertainties into account

-	These uncertainties increase with decreasing range lengths

Table 5.1.4.8-6: Measurement results for CFFdeltaNF methodology using black-box approach with changes in antenna array configuration from 8x8 (high-UL power) to OxP (low-UL power).

| Range Length | Description of Meas./Calc. | High-UL Power | Low-UL Power |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 8x8 | 8x4 @ (1,1) | 8x4 @ (1,2) | 8x4 @ (1,3) | 8x2 @ (1,1) | 8x2 @ (1,2) | 8x2 @ (1,3) | 8x2 @ (1,4) | 8x1 @ (1,1) | 8x1 @ (1,2) | 8x1 @ (1,3) | 8x1 @ (1,4) |
| FF @ 84cm | EIRP in FF BP Dir. [dB] | 45.61 | 40.64 | 40.84 | 40.99 | 37.18 | 35.61 | 34.96 | 36.46 | 33.87 | 31.9 | 31.43 | 30.18 |
| NF @ 35cm | EIRP in NF BP Dir. [dB] | 52.93 | 48.04 | 48.32 | 48.29 | 44.62 | 43.31 | 42.7 | 44.1 | 41.02 | 39.62 | 39.24 | 38.71 |
|  | Correlation Factor [dB] | -7.32 |  |  |  |  |  |  |  |  |  |  |  |
|  | Estimated FF EIRP in FF BP. Dir. [dBm] |  | 40.72 | 41.00 | 40.97 | 37.30 | 35.99 | 35.38 | 36.78 | 33.70 | 32.30 | 31.91 | 31.39 |
|  | FF Error in estimated EIRP [dB] |  | 0.08 | 0.16 | -0.02 | 0.12 | 0.38 | 0.42 | 0.32 | -0.17 | 0.40 | 0.49 | 1.21 |
| FF @ 84cm | EIRP in FF BP Dir. [dB] | 45.61 | 40.64 | 40.84 | 40.99 | 37.18 | 35.61 | 34.96 | 36.46 | 33.87 | 31.9 | 31.43 | 30.18 |
| NF @ 20cm | EIRP in NF BP Dir. [dB] | 56.33 | 52.13 | 51.97 | 52.53 | 48.41 | 46.83 | 46.61 | 48.04 | 44.56 | 43.32 | 42.33 | 42.95 |
|  | Correlation Factor [dB] | -10.72 |  |  |  |  |  |  |  |  |  |  |  |
|  | Estimated FF EIRP in FF BP. Dir. [dBm] |  | 41.41 | 41.24 | 41.81 | 37.68 | 36.11 | 35.89 | 37.31 | 33.84 | 32.60 | 31.61 | 32.23 |
|  | FF Error in estimated EIRP [dB] |  | 0.77 | 0.40 | 0.82 | 0.50 | 0.50 | 0.92 | 0.85 | -0.03 | 0.70 | 0.18 | 2.05 |

The next part focuses on simulations based on the CFFdeltaNF methodology using black-box approach, specifically the correlation factor approach where the low-UL power operation is no longer triggered. Instead, the low-UL power measurement is the result of performing measurements at frequencies that yield low EIRPs when compared to the EIRPs at the reference frequency, e.g., ACLR. Therefore, the test procedure for this approach is as follows with the first 4 steps identical and slightly adjusted steps 5 and 6 to the approach introduced earlier:

-	Step 1: Measure FF EIRP of the DUT in the FF beam peak direction for high-UL power with the FF probe at the reference frequency

-	Step 2: Perform a local search around the FF beam peak direction for high-UL power with the NF probe without compensating the probe antenna pattern at the reference frequency.

-	Step 3: Measure NF EIRP of the DUT in the NF beam peak direction (determined in Step 2) for high-UL power with the NF probe at the reference frequency

-	Step 4: Determine the correlation factor (difference between FF and NF EIRP measurements in Steps 1 and 3) and add the pathloss differences at the desired frequency

-	Step 5: Measure NF EIRP of the DUT in the NF beam peak direction with the NF probe at the desired frequency other than the reference frequency

-	Step 6: Estimate the FF EIRP for the desired frequency with the correlation factor determined in Step 4 and the NF EIRP measured in Step 5

The simplified PA model y= a1x+ a3x3 described in the previous section is used in this simulation. The 1st item a1k=1Ny⋅NzxkFkθk,ϕkFpαk,βkλ4πdkej2πdkλ is the major component for in-band (wanted) signal, while some of the 2nd item a3k=1Ny⋅Nzxk3Fkθk,ϕkFpαk,βkλ4πdkej2πdkλ will also contribute to the in-band pattern; the OOB signal’s pattern shape will be impacted by the non-linearity component a3xk3 [8][9].

When xk is defined in amplitude and phase expression as xk=xkexpjφk, we assume the amplitude is constant and only adjust the phase for each channel, then the 2nd item can be expressed as a3k=1Ny⋅Nzexpj3φkFkθk,ϕkFpαk,βkλ4πdkej2πdkλ

If  φk is not always zeros for all indices k , k=1Ny⋅Nzexpj3φkFkθk,ϕkFpαk,βkλ4πdkej2πdkλ will not generate the same pattern shape as k=1Ny⋅NzexpjφkFkθk,ϕkFpαk,βkλ4πdkej2πdkλ because of the different phase offset among multiple channels.

Below Figures 5.1.4.8-4 and 5.1.4.8-5 use the 8x2 antenna array with random position offset and beam tilt as an example to show the PA’s non-linearity impact on the OOB signal pattern shape.

For Figure 5.1.4.8-4, when the PA has poor linearity, the OOB signal pattern shape (plot on right) is different from the in-band signal pattern (plot on left). In this case, when using the relative correlation factor approach and the local NF peak direction deduced from the in-band signal to estimate OOB EIRP, the error can be as large as -3.67 dB.

For Figures 5.1.4.8-4, when the PA has very good linearity, the OOB signal pattern shape (plot on right) is almost same as the in-band signal pattern (plot on left). In this case, the error is insignificant.

These analyses show the out-of-band signal’s pattern shape will be impacted by the PA behaviour; the relative correlation factor deduced from the in-band signal to estimate out of band signal’s FF power will introduce extra errors. Clearly, the corresponding EIRP MUs are dependent on the PA and needs further study.

Figure 5.1.4.8-4: PA with poor linearity. In-band pattern on the left, OOB pattern on the right.

Figure 5.1.4.8-5: PA with good linearity. In-band pattern on the left, OOB pattern on the right.

The next investigation focuses on measurements and the 6-step procedure discussed earlier. The measurements presented in Table 5.1.4.8-7 use the same 8x8 antenna array introduced earlier. Here, a 200 MHz wide signal is applied to the RF ports of the phased antenna array and the power in the reference channel around 26 GHz as well as the adjacent channels (Low, High) were measured in both the NF at {20cm, 35cm} range lengths and FF at 84cm range length in the respective in-band beam peak directions.

Table 5.1.4.8-7: Measurement results for CFFdeltaNF methodology using black-box approach.

| Range Length [cm] | Beam Steering Angle [deg] ►►► | 0 | 10 | 20 | 30 | 40 | 50 | 60 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 35 | FF EIRP in reference channel at FF BP Direction [dBm] | 46.16 | 46.35 | 44.43 | 44.69 | 43.95 | 43.35 | 42.02 |
|  | NF EIRP in reference channel at NF BP Direction [dBm] | 52.05 | 51.89 | 50.92 | 51.83 | 51.32 | 50.82 | 49.42 |
|  | Correlation Factor [dB] | -5.89 | -5.54 | -6.49 | -7.14 | -7.37 | -7.47 | -7.40 |
|  | FF EIRP in reference channel (low) at FF BP Direction [dBm] | 24.49 | 25.12 | 22.03 | 24.22 | 22.48 | 21.39 | 20.24 |
|  | FF EIRP in reference channel (high) at FF BP Direction [dBm] | 27.68 | 28.86 | 26.23 | 26.50 | 26.83 | 25.76 | 25.31 |
|  | NF EIRP in reference channel (low) at NF BP Direction [dBm] | 30.71 | 30.22 | 28.25 | 30.79 | 28.94 | 28.07 | 26.69 |
|  | NF EIRP in reference channel (high) at NF BP Direction [dBm] | 32.82 | 33.54 | 32.25 | 32.53 | 32.86 | 31.87 | 31.09 |
|  | Estimated FF EIRP in reference channel (low) at FF BP Direction [dBm] | 24.82 | 24.68 | 21.76 | 23.64 | 21.57 | 20.61 | 19.29 |
|  | Error in estimated EIRP in reference channel (low) [dB] | -0.33 | 0.44 | 0.27 | 0.58 | 0.92 | 0.78 | 0.95 |
|  | Estimated FF EIRP in reference channel (high) at FF BP Direction [dBm] | 26.93 | 28.00 | 25.76 | 25.39 | 25.48 | 24.40 | 23.69 |
|  | Error in estimated EIRP in reference channel (high) [dB] | 0.75 | 0.86 | 0.48 | 1.11 | 1.34 | 1.35 | 1.62 |
| 20 | FF EIRP in reference channel at FF BP Direction [dBm] | 46.16 | 46.35 | 44.43 | 44.69 | 43.95 | 43.35 | 42.02 |
|  | NF EIRP in reference channel at NF BP Direction [dBm] | 55.79 | 56.48 | 55.79 | 56.05 | 55.90 | 55.27 | 55.31 |
|  | Correlation Factor [dB] | -9.64 | -10.13 | -11.36 | -11.36 | -11.95 | -11.92 | -13.29 |
|  | FF EIRP in reference channel (low) at FF BP Direction [dBm] | 24.49 | 25.12 | 22.03 | 24.22 | 22.48 | 21.39 | 20.24 |
|  | FF EIRP in reference channel (high) at FF BP Direction [dBm] | 27.68 | 28.86 | 26.23 | 26.50 | 26.83 | 25.76 | 25.31 |
|  | NF EIRP in reference channel (low) at NF BP Direction [dBm] | 33.15 | 35.01 | 32.97 | 35.01 | 34.04 | 32.84 | 31.80 |
|  | NF EIRP in reference channel (high) at NF BP Direction [dBm] | 42.18 | 43.44 | 42.04 | 42.15 | 42.48 | 41.16 | 37.29 |
|  | Estimated FF EIRP in reference channel (low) at FF BP Direction [dBm] | 23.52 | 24.88 | 21.61 | 23.64 | 22.09 | 20.92 | 18.51 |
|  | Error in estimated EIRP in reference channel (low) [dB] | 0.97 | 0.24 | 0.42 | 0.58 | 0.39 | 0.46 | 1.73 |
|  | Estimated FF EIRP in reference channel (high) at FF BP Direction [dBm] | 32.54 | 33.31 | 30.68 | 30.79 | 30.53 | 29.24 | 24.00 |
|  | Error in estimated EIRP in reference channel (high) [dB] | -4.86 | -4.45 | -4.45 | -4.29 | -3.71 | -3.49 | 1.31 |

These results show that the relative correlation factor approach of CFFdeltaNF is generally applicable for measurements at frequencies other than the reference frequency. However, as outlined earlier, MUs should be considered for this approach to take into account pattern changes due to non-ideal PA behaviour which was verified using measurements. More detailed analyses are required to study the non-linearity effects of PAs on the corresponding EIRP MU of CFFdeltaNF.

Another investigation focused on the effect of the UE pattern change over frequency in the resulting EIRP using the CFFdeltaNF methodology with black-box approach as described in clause 5.1.4.2. A simulation campaign using Matlab was defined following the assumptions presented in Table 5.1.4.3-1 with additional frequencies at 50, 100, 200, 400, 800 and 1200MHz from the centre frequency (i.e. 28GHz).

In this case, the EIRP error is calculated between the theoretical EIRP value in FF at the test frequency (e.g. 400MHz from channel’s centre frequency) and the resulting EIRP value calculated in the FF at the same test frequency but applying the ∆NFtoFF calculated for the reference test case at the reference frequency.

Mean and standard deviation errors, for all offsets and test frequencies, are presented in table 5.1.4.8-8, including results for different grid steps for the local search:

Table 5.1.4.8-8: Statistical results of EIRP simulations for CFFdeltaNF with black-box approach

| Antenna configuration | Range length (m) | Grid step | ¦Mean Error¦ (dB) | Std. Deviation (dB) |
| --- | --- | --- | --- | --- |
| 8x2 | 0.20 | 1º | 0.016 | 0.017 |
|  |  | 5º | 0.024 | 0.028 |
|  |  | 10º | 0.050 | 0.075 |
|  | 0.25 | 1º | 0.009 | 0.008 |
|  |  | 5º | 0.015 | 0.018 |
|  |  | 10º | 0.037 | 0.056 |
|  | 0.30 | 1º | 0.006 | 0.005 |
|  |  | 5º | 0.012 | 0.014 |
|  |  | 10º | 0.032 | 0.051 |
|  | 0.35 | 1º | 0.004 | 0.004 |
|  |  | 5º | 0.010 | 0.012 |
|  |  | 10º | 0.031 | 0.048 |
|  | 0.40 | 1º | 0.003 | 0.003 |
|  |  | 5º | 0.008 | 0.011 |
|  |  | 10º | 0.028 | 0.043 |
|  | 0.45 | 1º | 0.003 | 0.002 |
|  |  | 5º | 0.007 | 0.009 |
|  |  | 10º | 0.026 | 0.041 |

These results, assuming no change in antenna patterns due to non-linearities of the PAs, etc., confirm that CFFdeltaNF method is not impacted the potential changes in the pattern of the UE array due to frequency response.

#### 5.1.4.9	Simulation results for Influence of Noise

This clause provides influence of noise simulation assumptions and results for IFF/DFF, CFFDNF, and CFFNF methodologies.

The influence of noise quantifies the effect of a SNR at the TE input on EIRP measurements as outlined in [7].

B.2.1.27 Influence of noise This contributor describes an offset uncertainty factor caused by a noise floor especially in a case of low SNR. This contributor works as a bias to measured results only to a direction to increase values and thus this shall be included in the uncertainty budget table as a systematic uncertainty. The uncertainty value can be derived by the following equation. Influence of noise=10log101+10-SNR10

For low UL power test cases, the ~1m path losses due to range length (DFF) or focal distance (IFF) yield very poor SNR conditions at the TE input. Assuming a fixed noise level at the TE input, a reduction in measurement distance/range loss will significantly improve the SNR conditions for CFFDNF and CFFNF when compared to DFF/IFF as illustrated in Figure 5.1.4.9-1.

Figure 5.1.4.9-1: SNR Conditions for different test methodologies

The analyses in this clause are focused on the following PC3 assumption:

-	For an r1=20cm CFFNF range length (distance between probe and centre of QZ), the min distance between the probe and the antenna array is dCFFNF,min= r1-12.5cm (max offset)=7.5cm

-	For an r1=20cm CFFNF range length, the max distance between the probe and the antenna array is dCFFNF,max=r1=20cm

-	Similarly, for the CFFDNF methodology with a range length of rCFFDNF=35cm, the min (max) distance between the probe and the antenna array is dCFFDNF,min = rCFFDNF-12.5cm=22.5cm (dCFFDNF,max = rCFFDNF=35cm).

-	For the DFF/IFF calculations, we considered a rDFF/IFF=1m range length for simplicity.

The improvements in SNR for CFFNF and CFFDNF compared to DFF/IFF are tabulated in Table 5.1.4.9-1.

Table 5.1.4.9-1: SNR Improvement due to reduced measurement distance w.r.t. 1m DFF/IFF FSPL with fixed noise at TE Input

| Methodology ►► | DFF/IFF | CFFDNF |  | CFFNF |  |
| --- | --- | --- | --- | --- | --- |
| ▼f [GHz]▼ | @100cm | @35cm | @22.5cm | @20cm | @7.5cm |
| 24 | 0.0 | 9.1 | 13.0 | 14.0 | 22.5 |
| 30 | 0.0 | 9.1 | 13.0 | 14.0 | 22.5 |
| 35 | 0.0 | 9.1 | 13.0 | 14.0 | 22.5 |
| 40 | 0.0 | 9.1 | 13.0 | 14.0 | 22.5 |
| 45 | 0.0 | 9.1 | 13.0 | 14.0 | 22.5 |
| 50 | 0.0 | 9.1 | 13.0 | 14.0 | 22.5 |

In our influence of noise calculations, we assumed a range of SNR values for the DFF/IFF methodology at the TE input and scaled the effective SNR at the TE input for the CFFDNF/CFFNF analyses as shown in Table 5.1.4.9-2 based on the FSPL/SNR improvements in Table 5.1.4.9-1.

Table 5.1.4.9-2: Effective SNRs at the TE Input based on assumed SNR at the TE input for DFF/IFF

| Methodology ►► | DFF/IFF | CFFDNF |  | CFFNF |  |
| --- | --- | --- | --- | --- | --- |
| ▼DFF/IFF SNR [dB] ▼ | @100cm | @35cm | @22.5cm | @20cm | @7.5cm |
| -15 | -15.0 | -5.9 | -2.0 | -1.0 | 7.5 |
| -10 | -10.0 | -0.9 | 3.0 | 4.0 | 12.5 |
| -5 | -5.0 | 4.1 | 8.0 | 9.0 | 17.5 |
| 0 | 0.0 | 9.1 | 13.0 | 14.0 | 22.5 |
| 5 | 5.0 | 14.1 | 18.0 | 19.0 | 27.5 |
| 10 | 10.0 | 19.1 | 23.0 | 24.0 | 32.5 |
| 15 | 15.0 | 24.1 | 28.0 | 29.0 | 37.5 |

The analyses of the difference in EIRP when compared to the FF EIRP due to noise were based on 10k different AWGN simulations and N=30 averages were taken for each EIRP analysed. In each of the 10k AWGN simulations, a signal is generated first with 1000 samples, subsequently AWGN with specified SNR on the signal is applied, and in the end power of signal + AWGN was measured. These simulated results, labelled ‘|Mean Err to FF Reference|’ in the following tables are then compared with the calculations using the analytical 10*log1+10-SNR/10 equation and labelled ‘Influence of Noise’ in the following tables: for rDFF/IFF, dCFFDNF,min, and dCFFNF,min in Table 5.1.4.9-3 and for rDFF/IFF, dCFFDNF,max, and dCFFNF,max in Table 5.1.4.9-4. In these simulations, it was assumed that r2 = r1 + 2cm.

Table 5.1.4.9-3: Influence of Noise Simulations and Calculations for DFF/IFF, CFFDNF, CFFNF based on shortest measurement distance dmin

| DFF/IFF |  |  | CFFDNF |  |  | CFFNF |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| rDFF/IFF=100cm |  |  | dCFFDNF,min=22.5cm |  |  | (dCFFNF,min, dCFFNF,min+2cm) =(7.5, 9.5)cm |  |  |  |
| SNR @ rDFF/IFF (dB) | ¦Mean Err to FF Reference¦ (dB) | Influence of Noise (dB) | SNR @ dmin (dB) | ¦Mean Err to FF Reference¦ (dB) | Influence of Noise (dB) | SNR @ dmin (dB) | SNR @ dmin+2cm (dB) | ¦Mean Err to FF Reference¦ (dB) | Influence of Noise with SNR@ dmin +2cm (dB) |
| -15.00 | 15.1 | 15.1 | -2.0 | 4.0 | 4.2 | 7.5 | 5.4 | 1.44 | 1.1 |
| -10.00 | 10.4 | 10.4 | 3.0 | 1.6 | 1.8 | 12.5 | 10.4 | 0.45 | 0.4 |
| -5.00 | 6.2 | 6.2 | 8.0 | 0.5 | 0.6 | 17.5 | 15.4 | 0.09 | 0.1 |
| 0.00 | 3.0 | 3.0 | 11 | 0.1 | 0.2 | 22.5 | 20.4 | 0.04 | 0.0 |
| 5.00 | 1.2 | 1.2 | 17 | 0.1 | 0.1 | 27.5 | 25.4 | 0.07 | 0.0 |
| 10.00 | 0.4 | 0.4 | 23 | 0.1 | 0.0 | 32.5 | 30.4 | 0.09 | 0.0 |
| 15.00 | 0.1 | 0.1 | 28 | 0.1 | 0.0 | 37.5 | 35.4 | 0.09 | 0.0 |

Table 5.1.4.9-4: Influence of Noise Simulations and Calculations for DFF/IFF, CFFDNF, CFFNF based on largest measurement distance dmax

| DFF/IFF |  |  | CFFDNF |  |  | CFFNF |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| rDFF/IFF=100cm |  |  | dCFFDNF,max=35cm |  |  | (dCFFNF,max, dCFFNF,max+2cm) =(20, 22)cm |  |  |  |
| SNR @ rDFF/IFF (dB) | ¦Mean Err to FF Reference¦ (dB) | Influence of Noise (dB) | SNR @ dmax (dB) | ¦Mean Err to FF Reference¦ (dB) | Influence of Noise (dB) | SNR @ dmax (dB) | SNR @ dmax+2cm (dB) | ¦Mean Err to FF Reference¦ (dB) | Influence of Noise with SNR@ dmax +2cm (dB) |
| -15.00 | 15.1 | 15.1 | -5.9 | 6.8 | 6.9 | -1.0 | -1.8 | 5.7 | 4.0 |
| -10.00 | 10.4 | 10.4 | -0.9 | 3.4 | 3.5 | 4.0 | 3.2 | 2.7 | 1.7 |
| -5.00 | 6.2 | 6.2 | 4.1 | 1.4 | 1.4 | 9.0 | 8.2 | 1.0 | 0.6 |
| 0.00 | 3.0 | 3.0 | 9.1 | 0.4 | 0.5 | 14.0 | 13.2 | 0.4 | 0.2 |
| 5.00 | 1.2 | 1.2 | 14.1 | 0.1 | 0.2 | 19.0 | 18.2 | 0.1 | 0.1 |
| 10.00 | 0.4 | 0.4 | 19.1 | 0.0 | 0.1 | 24.0 | 23.2 | 0.0 | 0.0 |
| 15.00 | 0.1 | 0.1 | 24.1 | 0.1 | 0.0 | 29.0 | 28.2 | 0.0 | 0.0 |

The simulated influence of noise, i.e., |Mean Err to FF Reference|, was determined for non-fixed distances d. Here, 1000 random offsets uniformly spaced from 0 to 12.5cm were simulated and the individual results were averaged to obtain the results tabulated in Table 5.1.4.9-5 for PC3 devices. The simulated influences of noise are within the results at the respective extremes presented in Table 5.1.4.9-3 (min offset) and Table 5.1.4.9-4 (max offset). Similar simulations were performed for PC1 devices with the 12x12 antenna configuration with maximum offset of 10cm and are tabulated in Table 5.1.4.9-6.

Table 5.1.4.9-5: Influence of Noise Simulations and Calculations for DFF/IFF, CFFDNF, CFFNF based on 1000 random offsets uniformly spaced within 12.5cm (PC3) in a single hemisphere and N=1.

| DFF/IFF |  |  | CFFDNF |  | CFFNF |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| rDFF/IFF=100cm |  |  | 22.5cm ≤dCFFDNF≤35cm |  | 7.5cm≤dCFFNF≤20cm r2=r1+2cm |  |  |
| SNR @ rDFF/IFF (dB) | ¦Mean Err to FF Reference¦ (dB) | Influence of Noise (dB) | Average SNR @ r1 (dB) | ¦Mean Err to FF Reference¦ (dB) | Average SNR @ r1 (dB) | Average SNR @ r2 (dB) | ¦Mean Err to FF Reference¦ (dB) |
| -15 | 15.1 | 15.1 | -4.3 | 5.6 | 2.5 | 1.2 | 3.6 |
| -10 | 10.4 | 10.4 | 0.7 | 2.6 | 7.5 | 6.2 | 1.5 |
| -5 | 6.2 | 6.2 | 5.7 | 1 | 12.5 | 11.2 | 0.5 |
| 0 | 3 | 3 | 10.7 | 0.3 | 17.5 | 16.2 | 0.2 |
| 5 | 1.2 | 1.2 | 15.7 | 0 | 22.5 | 21.2 | 0 |
| 10 | 0.4 | 0.4 | 20.7 | 0.1 | 27.5 | 26.2 | 0 |
| 15 | 0.1 | 0.1 | 25.7 | 0.1 | 32.5 | 31.2 | 0 |

Table 5.1.4.9-6: Influence of Noise Simulations and Calculations for DFF/IFF, CFFDNF, CFFNF based on 1000 random offsets uniformly spaced within 10cm (PC1) in a single hemisphere and N=1.

| DFF/IFF |  |  | CFFDNF |  | CFFNF |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| rDFF/IFF=100cm |  |  | 35cm ≤dCFFDNF≤45cm |  | 20cm≤dCFFNF≤30cm r2=r1+2cm |  |  |
| SNR @ rDFF/IFF (dB) | ¦Mean Err to FF Reference¦ (dB) | Influence of Noise (dB) | Average SNR @ r1 (dB) | ¦Mean Err to FF Reference¦ (dB) | Average SNR @ r1 (dB) | Average SNR @ r2 (dB) | ¦Mean Err to FF Reference¦ (dB) |
| -15 | 15.1 | 15.1 | -7.2 | 10.5 | -3.1 | -3.8 | 6.8 |
| -10 | 10.4 | 10.4 | -2.2 | 6.2 | 1.9 | 1.2 | 3.4 |
| -5 | 6.2 | 6.2 | 2.8 | 3.0 | 6.9 | 6.2 | 1.3 |
| 0 | 3 | 3 | 7.8 | 1.1 | 11.9 | 11.2 | 0.4 |
| 5 | 1.2 | 1.2 | 12.8 | 0.4 | 16.9 | 16.2 | 0.1 |
| 10 | 0.4 | 0.4 | 17.8 | 0.1 | 21.9 | 21.2 | 0.1 |
| 15 | 0.1 | 0.1 | 22.8 | 0.0 | 26.9 | 26.2 | 0.1 |

While these results compare the simulated Influence of Noise, i.e., Mean Err to FF Reference, for the different methodologies at fixed DFF/IFF SNRs based on a 1m free-space path loss, it should be pointed out that for the same SNR at rCFFDNF, the CFFDNF Influence of Noise is lower than the CFFNF Influence of Noise for the same SNR at r2,CFFNF < rCFFDNF.

#### 5.1.4.10	Simulation Results for offset error MU

The analyses in this clause are to determine the uncertainties on EIRP/EIS when the antenna offset is declared incorrectly, i.e., when the actual antenna offset deviates from the declared antenna offset. These analyses are based on differences in path losses between the declared and the actual offsets and the difference in compensated probe gains. Here, the following assumptions were made:

-	1000 random offsets (xoffset, yoffset, zoffset), illustrated with red dots in Figure 5.1.4.10-1, were simulated and each offset was considered the actual offset of the antenna array.

-	For each random offset, 1000 random declaration errors (xerror, yerror, zerror), illustrated with blue dots in Figure 5.1.4.10-1, were generated with a fixed radius from the actual offset

-	For each actual offset and for each of the declared offsets, the device orientation was calculated so that the NF probe is placed in the actual/declared NF beam peak direction

-	For each actual offset and for each of the declared offsets, the corresponding path losses between the (actual/declared) antenna offsets and the probe were determined

-	For each actual offset and for each of the declared offsets, the corresponding probe antenna gains were determined in the respective NF beam peak directions.

Figure 5.1.4.10-1: Illustration of simulation assumptions. The 1000 random offsets considered the actual antenna offsets shown on the left; 1000 random errors around each offset shown on the right.

The results for this analysis are tabulated in Table 5.1.4.10-1. Similar analyses from another company are included in Table 5.1.4.10-1 focusing only on the uncertainties based on the effect of the antenna offset error on the pathloss, i.e., without the probe antenna gain impact.

Table 5.1.4.10-1: Statistical results of 1M EIRP CFFDNF simulations to determine the effect of offset declaration error on EIRP/EIS

| Error in declared offset [cm] | Range Length [m] | Company A | Company B |  |
| --- | --- | --- | --- | --- |
|  |  | Std. Dev of EIRP [dB] | Mean EIRP error [dB] | Std. Dev of EIRP [dB] |
| 0.1 | 0.4 |  | 0.056 | 0.019 |
| 0.5 | 0.2 | 0.17 |  |  |
|  | 0.25 | 0.12 |  |  |
|  | 0.3 | 0.10 |  |  |
|  | 0.35 | 0.08 |  |  |
|  | 0.4 | 0.07 | 0.074 | 0.087 |
|  | 0.45 | 0.06 |  |  |
|  | 1 | 0.03 |  |  |
|  | 20 | 0.00 |  |  |
| 1 | 0.2 | 0.35 |  |  |
|  | 0.25 | 0.24 |  |  |
|  | 0.3 | 0.19 |  |  |
|  | 0.35 | 0.16 |  |  |
|  | 0.4 | 0.14 | 0.129 | 0.221 |
|  | 0.45 | 0.12 |  |  |
|  | 1 | 0.05 |  |  |
|  | 20 | 0.00 |  |  |
| 2 | 0.2 | 0.71 |  |  |
|  | 0.25 | 0.49 |  |  |
|  | 0.3 | 0.38 |  |  |
|  | 0.35 | 0.32 |  |  |
|  | 0.4 | 0.28 |  |  |
|  | 0.45 | 0.24 |  |  |
|  | 1 | 0.10 |  |  |
|  | 20 | 0.01 |  |  |

### 5.1.5	Applicability of NF methodologies

Here, the applicability of the NF methodologies considered, i.e., direct Near Field (DNF), Combined Far-Field/Direct Near Field (CFFDNF), Combined Far-Field/Near Field (CFFNF), and Combined far-field/delta-near-field (CFFdeltaNF), are further analysed.

The CFFNF with transform (e.g. asymptotic expansion transform) has the following applicability:

-	Beam peak searches and spherical coverage test cases are performed with black box approach using the FF probe. Performing these tests with the NF measurement probe would require the extensive black&white-box approach which is not deemed a feasible enhancement of the methodology.

-	The low UL power/high DL power EIRP/EIS test cases in known FF BP direction are applicable to the black-box approach using transform techniques:

-	Three radii approach (i.e.  local search on radius r1 and very localized searches at r2 and r3) can be used.

-	EIRP/EIS can be approximated very accurately with the NF probe at very close distances with optimized improvements in relaxations

-	22cm for PC3 with ~0.3dB standard deviation and ~0.1dB mean error (systematic uncertainty),

-	32cm for PC1 with ~0.6dB standard deviation and ~0.3dB mean error (systematic uncertainty).

-	The unknown antenna location can be estimated accurately which allows very accurate TRP measurements at very close distances with large improvement in relaxations and no additional MU (mean error and standard deviation).

-	an MU element related to estimated DUT antenna offset error is required

-	an MU element related to the sensitivity of the asymptotic expansion approach to relative measurement uncertainty is required

-	the Influence of Noise MU element needs to be revised for the asymptotic expansion approach

-	EIRP/EIS based test cases require the compensation of the path loss (with respect to the active antenna array) and the compensation of the probe antenna pattern

-	The low UL power/high DL power EIRP/EIS test cases in known FF BP direction are applicable to the black&white-box approach.

-	Two radii approach without local searches can be used.

-	EIRP/EIS can be approximated very accurately with the NF probe at very close distances with optimized improvements in relaxations.

-	22cm for PC3 with no additional MU (mean error and standard deviation).

-	32cm for PC1 with~0.1dB mean error (systematic uncertainty).

-	an MU element related to declared DUT antenna offset error is required

-	an MU element related to the relative measurement uncertainty on the asymptotic expansion approach is required

-	the Influence of Noise MU element needs to be revised for the asymptotic expansion approach

-	EIRP/EIS based test cases require the compensation of the path loss (with respect to the active antenna array) and the compensation of the probe antenna pattern

-	The low UL power TRP test cases are not applicable to transform approach (CFFNF) since that approach would be test time prohibitive. However, the known offset (empirical evaluation with black box approach or declared with black&white-box approach) can be compensated using CFFDNF approach to obtain very accurate TRP results at very close distances.

The CFFDNF has the following applicability:

-	Beam peak searches and spherical coverage test cases are performed with black box approach using the FF probe. Performing these tests with the NF measurement probe would require the extensive black&white-box approach which is not deemed a feasible enhancement of the methodology.

-	The low UL power/high DL power EIRP/EIS test cases in the known FF BP direction are applicable to the black&white-box approach.

-	A local search to determine the NF test direction and/or optimize EIRP/EIS is not required.

-	EIRP/EIS can be approximated very accurately in the NF, i.e., at

-	35cm for PC3 with an additional 0.1dB mean error (systematic error) due to reduced range length

-	45 cm for PC1 with an additional 0.5dB mean error (systematic error) due to reduced range length.

-	For PC3, TRP test cases do not require additional measurement uncertainty due to reduced range length for

-	range lengths exceeding 20cm if the path loss correction is applied for measurement grids with step size of at most 5o

-	range lengths exceeding 25cm if the path loss correction is applied for measurement grids with step size of at most 10o

- 	non-uniform measurement grid can be utilized if the path loss correction is applied for measurement grids to further reduce the number of grid points without any additional measurement uncertainty, i.e., using a constant step size grid of ==5o within a ±30o cone centred around the NF beam peak and a constant step size grid of ==15o outside that cone.

-	for range lengths exceeding 40cm if the path loss correction is not applied for measurement grids with step size of at most 10o

-	For PC1, TRP test cases do not require additional measurement uncertainty due to reduced range length for

-	range lengths exceeding 20cm if the path loss correction is applied for measurement grids with step size of at most 5o

- 	non-uniform measurement grid can be utilized if the path loss correction is applied for measurement grids to further reduce the number of grid points without any additional measurement uncertainty, i.e., using a constant step size grid of ==2.5o within a ±20o cone centred around the NF beam peak and a constant step size grid of ==10o outside that cone.

-	for range lengths exceeding 35cm if the path loss correction is not applied for measurement grids with step size of at most 5o

-	EIRP/EIS based test cases require the compensation of the path loss (with respect to the active antenna array) and the compensation of the probe antenna pattern

-	an MU element related to declared DUT antenna offset error is required

The CFFdeltaNF has the following applicability:

-	Beam peak searches and spherical coverage test cases are performed with black box approach using the FF probe. Performing these tests with the NF measurement probe would require the extensive black&white-box approach which is not deemed a feasible enhancement of the methodology.

-	The low UL power/high DL power EIRP/EIS test cases in the known FF BP direction are applicable to the black-box approach.

-	A local search to determine the NF test direction is required.

-	EIRP/EIS can be approximated very accurately in the NF, i.e., at

-	35cm for PC3 with an additional 0.14dB mean error (systematic error) due to reduced range length for a 1deg step size of the NF local search grid size (based on antenna pattern/configuration changes); the effect of pattern changes due to non-linear PA behaviour is FFS.

-	45cm for PC1 with an additional 0.5dB mean error (systematic error) due to reduced range length for a 1deg step size of the NF local search grid size (based on antenna pattern/configuration changes); the effect of pattern changes due to non-linear PA behaviour is FFS.

-	The low UL power TRP test cases are not applicable.

DNF has the following applicability:

-	Beam peak searches and spherical coverage test cases are not applicable for the black-box approach. An  extensive black&white-box approach would be required to perform these tests with the NF measurement probe. Given the complexities of the extensive black&white-box approach, DNF is not deemed a feasible enhancement of the methodology for conformance testing but it might be suitable during UE development phase.

-	The low UL power/high DL power EIRP/EIS test cases in the known FF BP direction are not applicable to the black box approach.

-	The applicability of the low UL power/high DL power EIRP/TRP/EIS test cases in the known BP direction and with the black&white-box approach is FFS.

The assumption for this “black & white box” testing approach is that the antenna phase centre offset for the antenna panel that corresponds to the FF beam peak is known and declared, i.e., following the “white box” approach discussed earlier. On the other hand, however, it is assumed that the geometric centre of the DUT is aligned with the centre of the QZ, i.e., following the “black box” approach. This approach would have the same advantages as the “black box” approach over the “white box” approach in terms of complexity, test time, MU, and improvements of the relaxations and is summarized in Table 5.1.5-1 below.

Table 5.1.5-1: Comparison between the “black box” and “black & white box” approaches

| Approach | Knowledge of FF BP Direction (from Meas.) | Declaration of Antenna Phase Centre Offset of Antenna yielding BP | Need for FF probes and UBF | Need for local searches around NF BP | Meas. at different Radii | Test Time Impact | Estimated maximum Improvement of Relaxation (NOTE 1) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CFFNF for EIRP/EIS using Black Box | Yes | No | Yes | Yes | Yes (x3 in NF) | Medium (local searches & 3 different radii) | ~14dB (for 20cm range length). |
| CFFNF for EIRP/EIS using Black & White Box | Yes | Yes | Yes | No | Yes (x2 in NF) | Low (2 different radii in fixed NF BP Direction) | ~14dB (for 20cm range length) |
| CFFDNF for TRP using Black Box | Yes | No | Yes | No | No | None | Without offset correction: ~10dB (for 32cm range length) |
| CFFDNF for TRP using Black &White Box | Yes | Yes | Yes | No | No | None | With offset correction: ~14dB (for 20cm range length) |
| CFFDNF for EIRP/EIS using Black &White Box | Yes | Yes | Yes | FFS | No | Depends on local search | With pathloss correction: ~9dB (for 35cm range length) |
| CFFdeltaNF for EIRP/EIS using Black Box | Yes | No | Yes | Yes | Yes (x1 FF, x2 NF) | Low (local search) | ~9dB (for 35cm range length) |
| NOTE 1: Improvement of relaxation is only considering Free Space Path Loss |  |  |  |  |  |  |  |

### 5.1.6	Improvement of permitted methods

Tables 5.1.6-1 and 5.1.6-2 below provide a preliminary list of potential improvement of permitted methods based on the analysis provided by one company and are applicable to the frequency range of 24.25 – 43.5 GHz.

Table 5.1.6-1: Summary of potential improvement of permitted methods by Tx test case (24.25 – 43.5 GHz)

| Clause | Requirement | Testability issue | Test Metric | Regulatory related | TS 38.521-2 Test Requirements | Potential improvement |
| --- | --- | --- | --- | --- | --- | --- |
| 6.3.1 | Minimum output power | Low UL power | EIRP (Link=TX beam peak direction, Meas=Link angle). | No | No relaxation for PC1. For other power classes, relaxation varies from 0dB to 13.5dB depending on the operating band and channel bandwidth. | Improvements remove required relaxations from TC |
| 6.3.2 | Transmit OFF power | Low UL power | TRP (Link=TX beam peak direction, Meas=TRP grid) | Yes | Relaxations for n257: 21.4dB @ 50MHz, 24.4dB @ 100MHz, 27.4dB @ 200MHz and 30.4dB @ 400MHz. Relaxations for n258 and n261: [21.4]dB @ 50MHz, [24.4]dB @ 100MHz, [27.4]dB @ 200MHz and [30.4]dB @ 400MHz. Relaxations for n260: [24.1]dB @ 50MHz, [27.1]dB @ 100MHz, [30.1]dB @ 200MHz and [33.1]dB @ 400MHz. | ~ 10dB for FR2a and FR2b |
| 6.5.1 | Occupied bandwidth | Low UL power | OBW (Link=TX beam peak direction, Meas=Link angle) | Yes | No relaxations for FR2a and FR2b | N/A for FR2a and FR2b |
| 6.5.2.3 | Adjacent channel leakage ratio | Low UL power | TRP (Link=TX beam peak direction, Meas=TRP grid). | Yes | Relaxation for n257, n258 and n261: 0dB, except for 200MHz (1.5dB in two test IDs) and 400MHz (between 0 and 5.5dB) | Improvements remove required relaxations from TC TC coverage is extended for FR2b |
| 6.5.3.2 | Additional spurious emissions | Low UL power | TRP (Link=TX beam peak direction, Meas=TRP grid). | Yes | Between 0.3dB and 13dB relaxation depending on the combination of NR Band and Protected band. | TBD |

Table 5.1.6-2: Summary of potential improvement of permitted methods by Rx test case (24.25 – 43.5 GHz)

| Clause | Requirement | Testability issue | Test Metric | Regulatory related | TS 38.521-2 Test Requirements | Potential improvement |
| --- | --- | --- | --- | --- | --- | --- |
| 7.4 | Maximum input power | High DL power | EIS (Link=RX beam peak direction, Meas=Link angle). | No | 26dB relaxation for 24.25 ~ 29.5 GHz and 34 dB relaxation for 37 ~ 40 GHz with respect to minimun requirements. | ~ 12dB for FR2a ~16dB for FR2b |
| 7.5 | Adjacent channel selectivity (case 1) | High DL power | EIS (Link=RX beam peak direction, Meas=Link angle) | Yes | 50MHz: 1.8dB relaxation for power in transmission BW and interferer for band n260. 100MHz: 4.8dB relaxation for power in transmission BW and interferer for band n260. 200MHz and 400MHz are deemed not testable. | Similar improvements as for TC 7.4 Improvements remove required relaxations from TC |
| 7.5 | Adjacent channel selectivity (case 2) | High DL power | EIS (Link=RX beam peak direction, Meas=Link angle) | No | Decision not test ACS case 2. | Interferer need ~ 15-22dB relaxation |
| 7.6.2 | In-band blocking | High DL power | EIS (Link=RX beam peak direction, Meas=Link angle) | Yes | 50MHz: 1.8dB relaxation for power in transmission BW and interferer for band n260. 100MHz: 4.8dB relaxation for power in transmission BW and interferer for band n260. 200MHz and 400MHz are deemed not testable. | Similar improvements as for TC 7.4 Improvements remove required relaxations from TC |
| 7.9 | Receiver spurious emissions | Low UL power | TRP (Link=TX beam peak direction, Meas=TRP grid). | Yes | Relaxations for n257, n258, n260, and n261: 10.2dB between 6-20GHz, 17.2dB between 20-40GHz and 33.1dB between 40GHz and the 2nd harmonic. Relaxations for other bands are still TBD. | TBD |

For a given test case, NF based solutions should only be considered if the improvement for current methods is not enough to remove the relaxations determined by RAN5.

## 5.2	Polarization basis mismatch between the TE and DUT

### 5.2.1	General

The investigation of polarization basis mismatch enhancements to the FR2 test methodology includes the following aspects:  scope of test cases with polarization basis mismatch between the TE and DUT issues, enhanced test methods, applicability of the enhanced test methods.

The initial testing methodology for FR2 UE RF requirement verification is defined in [3] and features a measurement antenna capable of

-	transmitting and receiving on two orthogonal polarizations

-	introducing linearly polarized downlink signals at the centre of the quiet zone one polarization at a time

-	measuring the total uplink signal power by combining the power measured by two orthogonally polarized antennas sequentially or

-	demodulating the signal received by a single polarization at a time.

Regarding polarization basis mismatch between the UE TE and DUT, there are two main issues:

-	DL polarization basis mismatch for EIRP measurement. The mismatch between the TE and UE may lead such UEs to disable a Tx chain associated with one DL polarization and may result in an EIRP measurement which fails to include the polarization gain at some test points;

-	UL polarization basis mismatch for demodulation. Some UE implementations may support uplink transmission diversity schemes which, although transparent to the specification, impact the demodulation performance when the UL signal is demodulated on just a single polarization.

Therefore, these two issues are addressed separated with different approach. The potential solutions to minimize the impact of polarization basis mismatch address two distinct goals:  to enhance the EIRP measurement of UEs with various capabilities, and to enhance the test equipment demodulation performance in the uplink, such that a test mode to disable Tx diversity at the UE is no longer necessary for the UE to perform conformance testing.

### 5.2.2	Enhanced test method for EIRP measurement

#### 5.2.2.1	TPMI method

Transmitted Matrix Precoding Indicator (TPMI) is the basis of codebook based transmission enabling multi-port antenna transmission. TPMI method is identified as applicable method to enhance EIRP measurement, which is able to activate dual polarization transmission in EIRP measurement. The applicability of this method is defined in Clause 5.2.2.2.

For FR2 UEs support the TPMI method, the precoding matrix W is given by Table 5.2.2.1-1 (same as Table 6.3.1.5-1 in TS 38.211 [4]). 2Tx TPMI index 2-5 can force UE single-layer transmission using two antenna ports. Among them, only TPMI index 2 is selected for EIRP measurement.

Table 5.2.2.1-1: Precoding matrix W for single-layer transmission using two antenna ports.

| TPMI index | W (ordered from left to right in increasing order of TPMI index) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 – 5 | 1210 | 1201 | 1211 | 121-1 | 121j | 121-j | - | - |

The permitted test methods (i.e. DFF, IFF and NFTF) in [3] are all applicable for TPMI method with the additional procedure that the UE should be configured with TPMI index and working at single-layer transmission using two antenna ports, before performing EIRP-based test procedures in Clause 5.2.1.3 in TR38.810 [3].:

-	Peak EIRP Measurement Procedure

-	TRP Measurement Procedure

-	TX Beam Peak direction search and EIRP Spherical Coverage

#### 5.2.2.2	Applicability of TPMI side condition method

TPMI is applicable for one layer transmission with multi-port antenna. In FR2, dual polarization can be regarded as dual antenna ports, so it is natural to activate dual polarization transmission with TPMI side condition in EIRP measurement procedure.  However, for TPMI supporting dual antenna ports, the number of SRS ports (nrofSRS-Ports) is configured as 2 for both one layer transmission with ‘full power transmission’ and two layers transmission with regular UL MIMO, as specified in clause 6.1 of TS 38.101-2 [2]:

For a UE that supports 'UL full power transmission' and is configured to transmit a single layer with nrofSRS-Ports = 2, the requirements for UL MIMO operation apply only when it is configured for any of its declared full power modes in IE FullPowerTransmission-r16 (as defined in TS 38.331[13]). For a UE configured to transmit 2 layers, transmitter requirements for UL MIMO operation apply when the UE transmits on 2 ports on the same CDM group. The UE may use higher MPR values outside this limitation.

Thus, TPMI method is applicable for the following FR2 UEs:

-	Rel-15 Coherent UE

-	Rel-16 Coherent UE

-	Rel-16 UE supporting UL full power transmission mode1 (ul-FullPowerTransmission = fullpowerMode1)

Other UEs are not applicable for TPMI based test method.

#### 5.2.2.3	Alternative test method

A number of open issues have been identified with the configuration of 2-port CSI-RS method, and a conclusion whether this method is a feasible enhancement is TBD.

### 5.2.3	Enhanced test method for UL demodulation measurement

#### 5.2.3.1	Test equipment Zero-forcing MIMO receiver

As an enhancement to the FR2 test equipment topology, it has been proposed to adopt a zero-forcing MIMO receiver architecture so that dual-polarization transmissions by the UE can be demodulated by the test equipment receiver.

Two methods of demodulation and EVM calculation were discussed, one utilized DMRS-based channel inversion (Method 1), and the other based on inversion of the LSE-estimate of the channel (Method 2).

##### 5.2.3.1.1	Method 1

###### 5.2.3.1.1.1	Method 1 description

The EVM calculation method for 2-layer measurements is shown in Figure 5.2.3.1.1.1-1 and for 1-layer measurements in Figure 5.2.3.1.1.1-2.

Figure 5.2.3.1.1.1-1: EVM calculation block diagram for 2-Layer UL MIMO

Figure 5.2.3.1.1.1-2: EVM calculation block diagram for 1-Layer

The TE receives signals from 2 different ports which are connected to the dual polarized measurement antenna in the FR2 test system.

For the 2-layer transmission case shown in Figure 5.2.3.1.1.1-1 a MIMO equalization step as described in section 5.2.3.1.1.2 is performed to separate the layers.

For the 1-layer transmission case shown in Figure 5.2.3.1.1.1-2 the signals from both measurement antenna polarizations are combined using maximum ratio combining as described in section 5.2.3.1.1.3.

Each layer is then processed as described in section 5.2.3.1.1.4 to receive the measurement results for each individual layer.

###### 5.2.3.1.1.2	Method 1 MIMO Equalization

The MIMO equalization is based only on reference signals (DMRS) without using any data symbols. In order to obtain comparable EVM results independent of the number of DMRS symbols per slot, only the first DMRS symbol in each slot is used.

Estimation of effective 2x2 channel matrix is a well known procedure if reference signals use different subcarriers, e.g. in case of DMRS antenna ports 0 and 2. In case that same subcarriers are used, e.g. DMRS antenna ports 0 and 1, a channel decomposition is necessary taking advantage of the orthogonal codes wf and wt and assuming identical channel coefficients for adjacent subcarriers of same CDM group.

Effective channel including the precoding matrix P is:

H=HP= h0,0h0,1h1,0h1,1

with

hn,ν=ynrν*|rν|2

where y denotes the received symbol on port index n and r the reference signal for layer index ν.

Since reference signals of a specific layer are transmitted only on subcarriers of one CDM group channel, interpolation is needed in order to obtain channel coefficients for all subcarriers. Channel interpolation is done using the channel coefficients of active CDM group in all other CDM groups.

The channel coefficients used to calculate the equalizer coefficients are obtained after channel smoothing in frequency domain by computing the moving average of interpolated channel coefficients. The moving average window size is 7. For subcarriers at or near the edge of allocation the window size is reduced accordingly.

The ZF equalizer coefficients are calculated as the inverse of the effective channel matrix, in general:

GZF=H-1

###### 5.2.3.1.1.3	Method 1 Maximum Ratio Combining

The channel estimation for maximum ratio combining is based only on reference signals (DMRS) w/o including any data symbols. One or all DMRS symbols of one slot can be considered, but in order to obtain comparable EVM results independent of number of DMRS symbols per slot, only first DMRS symbol is used.

Estimation of effective 2x1 channel is a well known procedure. In case of transmit diversity, the effective channel includes the precoding matrix P:

H=HP= h0h1

with

hn=ynr*|r|2

where y denotes the received symbol on port index n and r the reference signal.

Since reference signals are transmitted only on subcarriers of one CDM group, channel interpolation is needed in order to obtain channel coefficients for all subcarriers. Channel interpolation is done using the channel coefficients of active CDM group in all other CDM groups.

The channel coefficients used to calculate the equalizer coefficients are obtained after channel smoothing in frequency domain by computing the moving average of interpolated channel coefficients. The moving average window size is 7. For subcarriers at or near the edge of allocation the window size is reduced accordingly.

The ZF equalizer coefficients for maximum ratio combining are calculated as pseudo inverse of effective channel, in general:

GZF=H+=(HHH)-1HH

###### 5.2.3.1.1.4	Method 1 Layer processing

After performing either the MIMO equalization as described in section 5.2.3.1.1.2 or the maximum ratio combining as described in section 5.2.3.1.1.3, each layer is processed using the existing procedure as defined in Annex E of TS 38.521-1 [6].

Since the channel estimation is calculated only on first DMRS symbol an averaging including all 14 symbols of one slot, i.e. data and reference signals, is needed in order to minimize EVM. The averaging is achieved by the LS equalization method described for single layer in Annex E.3. of TS 38.521-2 [6].

MS(f,t) and NS(f,t) are processed with a least square (LS) estimator, to derive one equalizer coefficient per time slot and per allocated subcarrier. EC(f) is defined for each layer as:

ECν(f)=t=013NSν(f,t)*NSν(f,t)t=013MSν(f,t)*NSν(f,t)

With * denoting complex conjugation. EC(f) are used to equalize layer data symbols.

EVM equalizer spectral flatness is derived from equalizer coefficients for each layer as follows:

cν=ECν(f) gν,02+gν,12

##### 5.2.3.1.2	Method 2

###### 5.2.3.1.2.1	Method 2 description

The block diagram of this demodulation scheme for FR2 2L UL is shown in figure 5.2.3.1.2.1-1. Everything to the right of the FFT blocks is per SC, and therefore must be replicated for the entire UL allocation. Figure 5.2.3.1.2.1-2 shows the 2L demodulation calculator while suppressing sections that would be unused for single layer operation.

For the 2-layer transmission case shown in Figure 5.2.3.1.2.1-1 a MIMO equalization step as described in section 5.2.3.1.2.2 is performed to separate the layers.

For the 1-layer transmission case shown in Figure 5.3.2.1.2.1-2 the signals from both measurement antenna polarizations are combined using maximum ratio combining as described in section 5.2.3.1.2.3.

Figure 5.2.3.1.2.1-1: Demodulation scheme for FR2 2L UL

Figure 5.2.3.1.2.1-2: Demodulation scheme for 2L UL, used for single layer UL

###### 5.2.3.1.2.2	Method 2 MIMO Equalization

For the 2L UL MIMO EVM test case, the system is framed as:

y=H G W x+n

Here, H is the 2x2 OTA channel, G is the 2x2 diversity scheme that the UE uses, and W is the 2x2 precoder matrix. G and W are UE-resident as shown in Figure 5.2.3.1.2.2-1.

Figure 5.2.3.1.2.2-1: UE implementation model

Here, the effective channel (HGW) as seen by the TE can be represented by a 2x2 matrix for each subcarrier (‘SC’). The process of equalization is implemented by multiplying data collected simultaneously at the TE receiver ports ‘y’ by an equalization matrix ‘A’. A unique ‘A’ matrix is computed for each SC. Using the ZF equalization method, ‘A’ is generated as the inverse of the estimate ‘F’ of the effective channel HGW for that SC.

x=A.y

A=F-1; where F= (HGW)

For each SC, the train of reconstructed symbols and received symbols are jointly used to estimate F (effective channel, also per SC), by minimizing the LSE between the received symbols and those that are calculated from the estimated channel and the reconstructed symbols, over all the symbols in the measurement interval. This technique is a 2L generalization of the legacy method to estimate the legacy rank 1 channel. This LSE minimization procedure results in the following channel estimate:

F=kyk.xkH.kxkxkH-1

Where xk and yk are respectively the kth transmitted and received OFDM symbol in the measurement interval:

xk=x1,kx2,k; yk=y1,ky2,k

Note the strong resemblance to the expression for channel estimate for the single layer legacy demodulation case.

While A can be taken as the inverse of ‘F’ for the ZF equalizer, we show later that there is a more general form of A that helps streamline single layer and 2L treatments. Two aspects bear further consideration: the existence of F and the invertibility of ‘F’, see section 5.2.3.1.2.5. The detailed treatment logically derives that if either F or F-1 do not exist, it is because the UE cannot support an OTA channel (coded into matrix ‘H’) of rank2, and is unable to maintain valid 2L UL. Failure to calculate either of the quantities therefore is tied to poor UE implementation rather than TE limitation.

Noting that A is the inverse of the estimate of the product HGW, the equalization step can be written as:

x=A.y=A H G Wx+A H n≝x+A H n

The matrix equation can be resolved into 2 separate scalar equations corresponding to the two layers:

x1=x1+v1

x2=x2+v2

Where, x̂m represents equalized layer data ‘m’, xm represents reconstructed (ideal) layer ‘m’ data, and vm represents noise accompanying ideal layer data ‘m’ after equalization. Note that the transmitted symbols are recovered without phase or gain modification, albeit in the presence of accompanying noise (responsible for EVM degradation). This is the case for every allocated SC – when the symbol on each sub carrier on each layer is reproduced without gain modification as this method does, no flatness artefacts are introduced into the measurements.

###### 5.2.3.1.2.3	Method 2 Maximal Ratio Combining

The channel estimate remains of the same form as in the 2L case, but the dimensions are now 2x1 because x is now a scalar for the single layer case:

F=kyk.xkH.kxkxkH-1

Because x is a scalar, (xxH)-1 is merely the reciprocal of a scalar quantity. Consequently, there are no numerical corner cases pertaining to existence of the quantity to ponder.

Due to F not being square, the equalization matrix ‘A’ must be derived as the pseudo inverse of F, rather than a true inverse as in the 2L case:

ASL=FHF-1FH

This type of ZF equalizer implements MRC for the single layer case. Note that this expression simplifies to A=F-1 for the 2L case, and so, ‘ASL’ is a more general construction for ‘A’ that is applicable to both cases (single layer as well as 2L).

###### 5.2.3.1.2.4	Method 2 EVM equalizer flatness

In the legacy single layer UL case, the channel estimate for each SC was a (complex) scalar, so it was straightforward to evaluate equalizer flatness. In the 2L test case however, the channel equalizer ‘A’ consists of 4 elements, 2 for each layer. Framing A as a row vector:

A=A1TA2T=a11a12a21a22

For layer ‘m’, recall that the ZF equalization coefficients for each layer AmT are scaled to diagonalize the channel. There is no realistic bound on the relative ratios of the coefficients, but the coefficients together preserve any frequency domain shape of the channel. Therefore, a composite parameter cm for each layer ‘m’ can be used to evaluate the equalizer spectrum flatness, given by:

cm=AmT

Or

cm=am12+am22

The FR2 single layer case is merely a subset of the 2L case, where ‘m’ can take on just one value:  1.

###### 5.2.3.1.2.5	Method 2 channel invertibility considerations

Recall that F, the estimate of the effective channel for each sub carrier is derived as an averaging operation over multiple OFDM symbols:

F=kyk.xkH.kxkxkH-1

The existence of F depends on the sum matrix xxH being rank 2. Recall now that ‘xk’ is the train of constellation symbols per SC, for both layers, as embedded in the train of OFDM symbols. For pseudo-random data, the sum matrix xxH accumulates positive values along the diagonal, and random zero-mean values in off diagonal locations (cross correlation across independent data). As the number of OFDM symbols increases in the averaging interval, this matrix tends towards a multiple of an identity matrix. Could this matrix be rank 1 in some corner case? Mathematically, this can happen only if ‘x’ is the same (both layers) in a SC for all OFDM symbols in the averaging interval. This outcome is extremely improbable if not impossible due to reliance on scramblers and standardized pseudo-random data generators in the UE. One can therefore conclude that F exists if the TE can merely work out ‘xk’. Since the TE estimates ‘xk’ by decoding measured data (see figure 2.2.1-1), the TE still depends on the UE’s ability to transmit data with enough inter-layer isolation to allow reconstruction. If F does not exist or is ill conditioned, it must mean that the UE is unable to maintain valid 2L UL.

The invertibility of ‘F’ depends on the invertibility of the cross-correlation term yxH in the expression for ‘F’:

kyk.xkH=H.GW.kxk.xkH+knk.xkH

The first term inside the parenthesis (GWxxH) involves 2 unitary matrices multiplied by a strongly diagonal matrix (as discussed in the section about the existence of F), and so remains rank 2. The second term inside the parenthesis (nxH) is a measure of correlation between noise and data symbols.

‘Noise’ can be truly thermal (low output power cases) in which case the second term would tend towards an all-zero matrix and can be ignored. We can therefore conclude that in low output power cases, yxH is rank 2 (i.e it is invertible) if H is also rank 2. Now, H contains information about the UE antenna’s cross-pol isolation: H is strongly rank 2 for UE implementations with good cross-pol isolation. H loses rank or becomes ill-conditioned if the UE’s transmit chains suffer from high antenna correlation. If F is not invertible or is ill conditioned, the reason can be traced back to the UE’s inability to maintain valid 2L UL.

Alternatively, the ‘noise’ can be composed of third order non-linearity from a PA. This is the case for high output power cases. A PA can be approximated by the time domain normalized characteristic shown here (k3 > 0):

vo=vi-k3.vi.vi2

The cubic term retains strong correlation with the linear term, so the PA model’s cubic term output can be approximated as sum of a correlated component (‘a.x’, where ‘a’ is related to the correlation coefficient of the cubic term to the linear term) and an un-correlated component (‘nun’). In the post-FFT frequency domain, yxH can be rewritten as:

kyk.xkH=H.GW.kxk.xkH-α.k3.kxk.xkH+knunk.xkH

We focus on the matrix inside the parenthesis:

B=GW.kxk.xkH-α.k3.kxk.xkH+knunk.xkH

The third term inside the parenthesis is a measure of correlation between data and the un-correlated component, which (by definition) would tend towards an all-zero matrix and can be ignored, like the low output power case. There is still risk of yxH losing rank and becoming non-invertible, however, for example if the matrix (GW-a.k3.I) loses rank.

Note that this mechanism (3rd order nonlinearity) also exists in the case of demodulation by TE with single chain receivers, and experience tells us that PA non-linearity products do not contribute to a situation where the channel cannot be inverted. Legacy demodulation procedures therefore serve as empirical proof of the argument that the matrix (G.W-a.k3.I) never loses rank.

Physically, for compliant UEs, | k3| is in the range of 0.10-0.20 in normalized form, and a, while PAPR dependent, can be verified to be <= 2 for NR UL waveforms. We also note that matrices G and W are unitary, which allows direct comparison of the terms in (GW-a.k3.I);  the relative magnitude of | a.k3| is small enough that matrix B would still be dominated by the first term (GW xxH) for the range of EVMs expected to be measured. We therefore expect B is rank 2 for pseudo random data. Consequently, here too, rank of yxH depends solely on rank of H. i.e yxH is rank 2 (i.e it is invertible) if H is also rank 2. If F is still not invertible (i.e yxH is rank 1), it is because the UE cannot support an OTA channel (H) of rank2, and is unable to maintain valid 2L UL.

This treatment can conclude with the observation that both F and F-1 exist provided the UE can support a rank 2 OTA channel, i.e rank(H)=2.

###### 5.2.3.1.2.6	Method 2 DC (LO) cancellation

EVM calculation is gated by the TE’s procedure to cancel LO. Per existing signaling, only one LO location can be declared per BWP for the carrier being tested, be it single layer or two.  More significantly, the only locations allowed to be declared are SC locations. For NR, LO cancellation based on signaling declaration of LO location can therefore be performed either pre- or post-FFT. Per layer quantities are only available in the post-FFT, post-equalization domain where the channel has already been diagonalized, so the focus in the treatment below is the post-FFT method.

A UE can expect LO cancellation only if the LO location is signalled in terms of a valid subcarrier location. If the UE reports 3300 or 3301 via IE txDirectCurrentLocation the TE or network can get no information on LO location other than ‘it does not coincide with the FFT grid’. This aspect is consistent with the exception in the core-requirement pertaining to this IE.

The following passage describes a method based on post-FFT LO removal based on LO location declaration via IE txDirectCurrentLocation. Post-FFT LO removal involves removing dependency on the LO-bearing SC or being able to ignore the impact of LO leakage.

Recall that each allocated SC has an instantiation of the post-FFT procedure outlined in figure 2.2.1-2. The equalization matrix ‘A’ is calculated for each sub carrier and is derived from reconstructed symbols recovered from processing DMRS. In NR, DMRS symbols do not occupy each SC of the OFDM symbol, necessitating some form of interpolation for DMRS-less sub carriers. To minimize impact of LO leakage on the accuracy of the process of reconstructing the transmitted symbols, a logical choice is to select a DMRS comb that skips over the SC declared to have the LO leakage.

While it is relatively easy to ensure that the reconstruction process remains free of impact from LO leakage by using the comb, the measured symbols ‘y’ for the LO-bearing SC still contain a systematic and constant leakage term for all OFDM symbols in the measurement interval.

y=H G W x+n+ c1c2≝ yLO+C; where C≝Hc1c2

where c1 and c2 are complex constants pertaining to the LO leakage. The channel estimate for the LO-bearing SC starts with the LSE estimate:

F=kyk.xkH.kxkxkH-1

=C.kxkH.kxkxkH-1+kyLOk.xkH.kxkxkH-1

There are multiple options to deal with this extra term from LO leakage (C(xH(xxH)-1). The first option is to use a very long-term averaging to determine ‘F’ for the LO-bearing SC. Since ‘C’ is a constant and (xH) can reasonably be assumed to be zero for a long enough average, the contribution of the first term reduces to zero. So, the first option is to do nothing other than ensure a very long-term average is used for calculating ‘F’ for the LO bearing SC. The second option is to interpolate from neighbour SCs to determine F for the LO-bearing SC. Other, more sophisticated treatments may also exist. ‘F’ calculated from either of these methods can be compared to ‘F’ calculated using the LSE-estimate expression over the standard measurement interval to derive the value of ‘C’ for the measured interval.

The equalization equation for an LO-bearing SC is:

Figure 5.2.3.1.2.6-1: Equalization equation for an LO-bearing SC

The calculated quantity ‘AC’ can be used both to estimate carrier leakage per layer, as well as to correct the estimated symbol on the LO-bearing SC prior to EVM calculation.

##### 5.2.3.1.3 Analysis of Method 1 & 2

###### 5.2.3.1.3.1 Comparison of both Methods

To downselect one of the Methods described above, the EVM measurement performance for both methods has been analysed with the following assumptions:

-	Test waveforms constructed with:

-	Flat signal PSD

-	Injected AWGN

-	Signal configuration:

-	[50/66] contiguous RBs

-	CP-OFDM (QPSK – 64QAM – 256QAM) PUSCH, rank 2

-	UL RMC as defined in appendix of 38.101-2

In the following, the results of the analysis of Method 1 and 2 are presented and compared.

As a first step the EVM accuracy for both methods has been analysed. As can be seen from Figure 5.2.3.1.3.1-1, both Methods show good EVM measurement accuracy, while Method 1 has equivalent or superior accuracy to Method 2.

Figure 5.2.3.1.3.1-1: EVM accuracy evaluation for slot length signals with Method 1 & 2

Figure 5.2.3.1.3.1-2 shows further analysis of Method 1, here the results of 2-layer signal analysed with Method 1 are compared to an analysis of a SISO signal, which has been measured according to the EVM measurement definition in TS 38.521-2 Annex E. For signals with 3 DMRS as defined for the UL RMCs the difference is around 0.2 dB, This applies for all different modulations as can be seen from the graphs for QPSK and 64QAM.

Figure 5.2.3.1.3.1-2: Absolute EVM accuracy evaluation for slot length signals with Method 1

Further analysis of Method 1 shows a good performance also under the condition of varying antenna misalignments or antenna port configurations. While in Figure 5.2.3.1.3.1-3 it can be observed that for reasonable SNR levels, the EVM is only overestimated by less than 0.5 dB, this may still mean that the EVM evaluation is impacted by the different antenna ports or antenna misalignment. Figure 5.2.3.1.3.1-4 shows that the EVM evaluation is independent of those factors and the same results are obtained for different combinations of antenna ports and misalignments.

Figure 5.2.3.1.3.1-3: Method 1 EVM evaluation for the identity matrix (left) and rotation matrix (right)

Figure 5.2.3.1.3.1-4: Method 1 EVM results for different antenna misalignments and ports

###### 5.2.3.1.3.2 Conclusion

Based on the analysis shown in section 5.2.3.1.3.1 it is recommended to adapt Method 1 as the enhanced test method for UL MIMO demodulation measurement. Further implementation details to ensure that the results among different TE vendors are comparable, are up to RAN5 definition.

## 5.3	Inter-band (FR2+FR2) CA

### 5.3.1	General

In this sub-clause, following analyses with an in-direct far field (IFF) OTA test systems are introduced in achieving RF measurements of an inter-band CA UE in FR2.

a)	Impact of multiple test antennae

1)	PSD imbalance with DL signals from test equipment

DL PSD towards UE supporting independent beam management (IBM)

DL PSD towards UE supporting common beam management (CBM)

2)	Impact of off-focus test system antennae

Quality of quiet zone (QoQZ)

Rx beam profiles with an independent beam management (IBM) UE

Propensity to trigger incorrect beam in CBM UE

Spherical coverage measurement simulation with common beam management (CBM) UE

3)	Summary on applicability of offset antenna test system

b)	Inter-band testing ramifications

### 5.3.2	Impact of multiple test antennae

#### 5.3.2.1	PSD imbalance with DL signals from test equipment

##### 5.3.2.1.1	DL PSD towards UE supporting independent beam management (IBM)

An impact of an AoA offset ranging from 0 to 7 degrees was analyzed for UE supporting independent beam management.  The gain difference between the CCs is a test case parameter informed by the recent agreement on the PSD difference in the REFSENS and EIS spherical coverage requirement for DL inter-band CA within FR2 [2].  The simulation assumptions are shown in Table 5.3.2.1.1-1 below.

Table 5.3.2.1.1-1: Simulation assumptions for PSD imbalance

| Simulation Parameters |  |
| --- | --- |
| Antenna array size | 4x1 |
| Element spacing | 5mm |
| Element pattern | See TR38.803 |
| Antenna impairments | Not considered |
| Phase shifter impairments | See TR38.817-01 |
| Transmission line impairments | Modeled TL length and loss per element |
| AoA offset | {0, 2, 5, 7} deg |
| Beam management assumptions | Independent beam management on each CC |
| Center frequencies of component carriers | f1 = 27.9 GHz, f2 = 38.5 GHz |
| Gain difference between CC1 and CC2 | 15.2 dB |

For each AoA offset the PSD difference between CC1 and CC2 was calculated after spatially filtering the signal with the array response pattern corresponding to each CC.  Figure 5.3.2.1.1-1 below illustrates the simulation results.

Figure 5.3.2.1.1-1: Spatially filtered PSD difference vs. angular separation between CCs

Table 5.3.2.1.1-2 below summarizes the simulation results.

Table 5.3.2.1.1-2: Simulation results for spatially filtered PSD difference vs. angular separation between CCs

| Max excess PSD difference due to AoA offset (dB) |  |
| --- | --- |
| AoA offset (º) | PSD diff (dB) |
| 2 | 0.4 |
| 5 | 1.1 |
| 7 | 1.7 |

The requirement for inter-band CA between bands n261 and n260 is defined assuming a non-zero PSD difference between the component carriers.  This assumption holds for the class of inter-band CA configurations to which the independent beam management (IBM) type is applicable. It is observed that at least for IBM inter-band CA requirements, AoA offsets of up to 7 degrees between two FR2 CA component carriers do not significantly impact the PSD difference assumption taken for the core requirement. This observation should be double-checked after the band n262 requirements are finalized and the scope of CA configurations with n262 are understood.

Further note that as part of the Rel-17 FR2 RF enhancement work item, RAN4 is discussing the potential definition of requirements assuming common beam management (CBM) for combination of certain band groups, such as 28+28 GHz.  An analysis of the impact of AoA offsets in the test setup for inter-band CA with CBM is needed after the scope of CBM requirements and associated agreements are better understood.

##### 5.3.2.1.2	DL PSD towards UE supporting common beam management (CBM)

Following a similar simulation methodology as described in 5.3.2.1.1, an analysis of the impact of an AoA offset ranging from 0 to 7 degrees was analyzed for UE supporting common beam management.  The gain difference between the CCs is a test case parameter informed by the recent agreement on the PSD difference in the REFSENS and EIS spherical coverage requirement for DL inter-band CA within FR2 [2].  The simulation assumptions are shown in Table 5.3.2.1.2-1 below.

Table 5.3.2.1.2-1: Simulation assumptions for PSD imbalance with CBM

| Simulation Parameters |  |
| --- | --- |
| Antenna array size | 4x1 |
| Element spacing | 5mm |
| Element pattern | See TR38.803 |
| Antenna impairments | Not considered |
| Phase shifter impairments | See TR38.817-01 |
| Transmission line impairments | Modeled TL length and loss per element |
| AoA offset | {0, 2, 5, 7} deg |
| Beam management assumptions | Common beam management between CC1 and CC2 (codebook optimized for f1) |
| Center frequencies of component carriers | Case 1: f1 = 24.25 GHz, f2 = 29.5 GHz Case 2: f1 = 37.0 GHz, f2 = 43.5 GHz |
| Gain difference between CC1 and CC2 | Case 1: 0.0 dB Case 2: 1.2 dB (difference in EIS spherical coverage between n260 and n259) |

For each AoA offset the PSD difference between CC1 and CC2 was calculated after spatially filtering the signal with the array response pattern corresponding to each CC.  Figure 5.3.2.1.2-1 below illustrates the simulation results.

a)  b)

Figure 5.3.2.1.2-1: Spatially filtered PSD difference vs. angular separation between CCs; a) Case 1 (24.25 + 29.5 GHz); b) Case 2 (37.0 + 43.5 GHz)

Table 5.3.2.1.2-2 below summarizes the simulation results.

Table 5.3.2.1.2-2: Simulation results for spatially filtered PSD difference vs. angular separation between CCs

| Max excess PSD difference due to AoA offset (dB) |  |  |
| --- | --- | --- |
| AoA offset (º) | PSD diff (dB) |  |
|  | Case 1 | Case 2 |
| 2 | 0.3 | -0.2 |
| 5 | 0.8 | 0.3 |
| 7 | 1.7 | 1.2 |

Because RAN4 is still discussing the potential introduction of requirements for CBM CA between bands within the same band group (as of #98-e meeting 2021 Jan.), the PSD difference analysis in this subclause assumes a convergence toward 0 dB PSD difference or, in the case of bands with different spherical coverage requirements, for the difference to be bounded by the difference in spherical coverage EIS values.

We note that because the CBM CA architecture is, in essence, an optimization, the UE receiver is more sensitive to PSD differences beyond those assumed for the core requirement.  Although Table 5.3.2.1.2-2 indicates that the maximum excess PSD difference due to AoA offset ≤ 1.7 dB which is applicable only to PC3 UE, this effect compounds with the beam squint impairment.

Again noting that the core requirement work on CBM CA is still ongoing, it is not possible to use the results to disqualify the proposed AoA offset method from applicability to CBM CA test cases. However, the impact of AoA offset on PSD difference assumptions made in the core requirement definition should be taken into account.

#### 5.3.2.2	Impact of off-focus test system antennae

##### 5.3.2.2.1	Quality of quiet zone (QoQZ)

The IFF system for FR2 UE OTA testing is constructed around an offset parabolic mirror to collimate beams from a test antenna towards the UE. The architecture allows for a quiet zone (QZ) roughly the size of the spot on the mirror illuminated by the source. The key to this architecture is locating the source at the focus of the parabola that describers the mirror.

The burgeoning number of FR2 bands place increasing demands on both, the antenna, as well as the RFFE in test equipment.

The effect of off-focus test system antenna in IFF systems was studied before listing ramifications to inter-band test requirements. The components of the IFF are much larger than the wavelength of signals under test. The primary effect of the EM solution to the IFF problem can hence be determined by an equivalent optics problem.

From the geometry of a parabola, there is a unique location (the focus) that allows a test antenna to appear as a far-field antenna after reflection off the mirror. The far-field characteristic comes from the wavefront geometry that is transformed by the mirror from spherical to planar. The planar wavefront is normal to the optical axis of the mirror. Unfortunately, the favorable characteristics of the parabolic mirror are limited to proper placement of the source, and optical aberration is expected outside the geometry demands specific to the mirror in use. For example, in astronomy, the ‘coma’ aberration has long been associated with telescopes when wavefronts of incoming light are not normal to the optical axis.

To simplify the problem, the following assumptions were made about the IFF system, which was modelled with a 2D ray tracer:

1.	The TE antenna was modelled as a point light source with uniform illumination in the hemisphere facing the mirror, and no illumination facing the source.

2.	Light not incident on the mirror was modelled as being perfectly absorbed by the background

3.	Edge effects of the mirror were not considered

4.	The primary focal length was chosen to be 0.65m, for an effective focal length with the offset mirror of ~ 0.7m. The mirror offset was such that the mirror extended from y = 0.2 m to y = 0.5 m off the optical axis.

5.	QZ plane was placed orthogonal to the optical axis 1.5 m away from the point where the axis intersects the parabola.

6.	Source-to-source interaction, in case of multiple sources, are neglected

7.	EM field perturbation due to presence of positioner or DUT fixturing is neglected by virtue of choosing an optic method

8.	Light source locations considered (d,h defined in figure 5.3.2.2.1-1):

Note that assumption #1 is not valid for typical horn antennae, but the uniform illumination assumption serves as a good reference point for studying power variation across the QZ when the source is moved off-focus.

Table 5.3.2.2.1-1: Source locations

| Location id | d (m) | h (m) |
| --- | --- | --- |
| 1 (focus) | 0 | 0 |
| 2 | 0 | 0.05 |
| 3 | 0 | 0.10 |
| 4 | -0.05 | 0.10 |
| 5 | +0.05 | 0.10 |

Figures 5.3.2.2.1-1 shows the position of the mirror relative to the sources, as well as the illumination levels at the QZ (sub figure 1b). The blue curve in the sub-figure 1b indicates 2 things about a source at the focus (location1). The first observation is that the nominal QZ is located between y = 0.2 and y=0.5, which coincides with the mirror extents in the y-dimension. This detail is expected. The second observation is that an isotropic source gets transformed to a non-uniform illumination at the QZ.

Figure 5.3.2.2.1-1: a: Mirror and source locations; b : Source to QZ power density distortion

Furthermore, the illumination intensity curves corresponding to other source locations inform that the collimated beam moves progressively off the QZ as the source is moved away from focus. In the example locations studied, the beam develops a down tilt, but an up tilt is also possible if the source is moved to a location, say, below and to the left of the focus in figure 1a.

In figure 5.3.2.2.1-2, it is observed that the wavefront also starts to deviate from being purely orthogonal to the optical axis. The wavefront shape is consistent with the ‘beam tilt’ observation made earlier.

Figure 5.3.2.2.1-2: Wavefront orientation at QZ due to off-focus source

For a UE’s perspective on off-focus sources, candidate source locations were characterized by their angular locations relative to mirror normal. Note that the focus is off the mirror normal due to ‘offset mirror’ geometry, so it has a non-zero value. In the example studied, the focus is 15.4 degrees below mirror normal, and the other off-focus source locations studied all have lower angular offsets than the focus. In figure 5.3.2.2.1-3 we show that the rate of increase beam tilt and the rate of increase in angular offset of the source are very similar, i.e close to 1:1 for the geometry studied.

Figure 5.3.2.2.1-3: Beam tilt due to off-focus source

Recall that the nominal QZ location is the illuminated spot in the plane of the QZ when the source is located at the focus of the mirror. In case of our example, the nominal QZ extends from (x=1.5, y= 0.2) to (x=1.5, y= 0.5). Beam tilt causes the illuminated spot in the plane of the QZ to move off the nominal QZ location. When multiple sources illuminate the mirror, the effective QZ size reduces to the common area across the beams from all sources. Beam tilt consequently has the effect of reducing the effective QZ size. See figure 5.3.2.2.1-4. The reduction in size is a function of both, the angular offset between source and focus, and the distance of the QZ from the mirror.

Figure 5.3.2.2.1-4: Beam tilt causes QZ size reduction

Now, different UE power classes have different beam shapes, and different beam packing densities. For power class 1, 3dB beam widths are expected to be between 5 and 10 degrees as an extreme example. A beam tilt in the range of a few degrees could cause a PC1 UE to select a different beam for an off-focus source, relative to a source at the focus.

UE size also limits how close to the mirror the QZ plane can be moved. The radiating face of PC1 device can measure in the 10s of cm in each dimension, which would force the QZ far away from the mirror. This restriction would, in turn, cause much greater reduction in QZ size for a given beam tilt angle, when using multiple sources. These problems would be less serious for PC3 devices.

In a case a test antenna is located simply off the position from a focal point (normally at a position of a main test antenna) in the IFF test system, a centre of a beam from the offset feed antenna would shift depending on a distance of focal length (f), a distance between the reflector and a centre of a quiet zone (r), and a distance of the antenna offset (δ). Figure 5.3.2.2.1-5 depicts the relationship of this shift (δ’).

Figure 5.3.2.2.1-5: Offset of antenna beam peak from centre of QZ (Top view of IFF test system)

Here the shift (δ’) can be calculated geometrically by the following equation, and it may cause an increase of QoQZ measurement uncertainty in a 30 cm quiet zone.

δ’= δ* rf-1#1

From our experience to date, an electric field intensity in a quiet zone when a feed antenna is located near a focal point is decided by two factors, an antenna pattern of the feed antenna for a test equipment and a shape of the reflector. Especially the antenna pattern can be assumed as the main factor to decide this characteristic, which is directly connected also to the QoQZ.

Based on this observation an estimation was made with an impact of the offset antenna to QoQZ from an experimental data which was obtained by measuring the electric field intensity of a feed antenna via a reflector. Figure 5.3.2.2.1-6 shows one of the experimental data obtained by scanning the field intensity in a range of +/- 200 mm from a centre of the quiet zone.  Here a 40.8 GHz vertical polarization beam was scanned along with theta (x) direction.

Figure 5.3.2.2.1-6: Electric field intensity of feed antenna scanned along with theta direction (40.8 GHz, V-pol)

Table 5.3.2.2.1-2 and Figure 5.3.2.2.1-7 show the estimation of difference between the QoQZ of main antenna and of the offset antenna. Note that these values are specific to the feed antenna (amplitude taper) in this experiment and thus they may vary depending on an antenna pattern used by each test equipment vendor.

Table 5.3.2.2.1-2: Estimation of QoQZ difference between main and offset antenna

|  | Estimation of QoQZ difference (EIRP) [dB] |  |  |
| --- | --- | --- | --- |
| δ [mm] | 23.45 GHz | 32.125 GHz | 40.8 GHz |
| 0 | 0.00 | 0.00 | 0.00 |
| 15 | 0.02 | 0.02 | 0.01 |
| 30 | 0.04 | 0.05 | 0.03 |
| 45 | 0.08 | 0.08 | 0.06 |
| 60 | 0.11 | 0.11 | 0.09 |
| 75 | 0.14 | 0.15 | 0.11 |

Figure 5.3.2.2.1-7: Plot of QoQZ difference

Though these differences may vary depending on an antenna pattern (amplitude taper) of a feed antenna, as can be seen from above, there is a chance that the difference of QoQZ due to the offset antenna can be limited within an acceptable level by optimizing an arrangement of antennas.

As described above, these QoQZ differences are estimated based on the antenna configuration with which the offset antenna is simply off the position from a focal point. And there is a way to mitigate the impact of the offset antenna to the actual QoQZ.

One of major factors to decide QoQZ is the electric field intensity in the quiet zone. Figure 5.3.2.2.1-8 depicts a 2D-image of the electric field intensity (amplitude taper) in the quiet zone when the offset antenna is used. As shown in the figure, due to the shift of beam centre from the main antenna, distribution of the field intensity becomes asymmetric in the quiet zone and thus it causes the increase of the QoQZ MU value. However it is possible to shift the beam peak position by tilting the offset antenna and make the distribution of the field intensity close to symmetric in the quiet zone like the one from the main antenna, allowing us to mitigate the impact of beam centre shift to the QoQZ value. Figure 5.3.2.2.1-9 depicts the image of the improvement with the offset antenna placement. At the same time, there might be diffraction and/or scattering effects created by the reflector’s paraboloid edges and the size of the paraboloid that impact  the QoQZ characteristics. Thus, there should be some limitations with antenna offset ranges and angles to tilt the offset antenna that depend on a relationship between reflector size, measurement antenna offset and range length. All these factors should be considered during the design of the test system.

Figure 5.3.2.2.1-8: 2D image of the electric field intensity (amplitude taper) in the quiet zone from an offset antenna

Figure 5.3.2.2.1-9: 2D image of the electric field intensity in the quiet zone from a tilted offset antenna

There is another way of recovering desired QZ illumination. In Figure 5.3.2.2.1-4, it is shown that beam tilt causes reduction in size of QZ that is common to all source locations. This mechanism is intuitive and can be pre-compensated during design of the test system.

The matter of QZ quality however involves more complexity. As a first approximation, QZ quality can be quantified by the illumination distribution from a hypothetical constant density source illuminating the mirror. Here it becomes evident that angular offset of the antenna alone is not enough to determine illumination distribution. See figure 5.3.2.2.1-10. Locations 2, 3 and 4 all have approximately the same angular offset (<0.5 degree difference), but their QZ illumination can be made better or worse than that of the on-focus source by adjusting mirror to source distance. In the graphic example, the source location closest to the mirror has the least variation QZ illumination.

It is therefore possible to recover desired QZ illumination by adjusting the source location distance from mirror in concert with angular offset. A discussion regarding whether the different QoQZ MU needs to be applied compared to the single carrier case can be left to RAN5.

As shown later in the discussion of wavefront shapes however (see sub-clause 5.3.2.2.3), there are other constraints governing offset source to mirror distance.

Figure 5.3.2.2.1-10: a: Mirror and source locations. Locations 2, 3 and 4 have similar offset; b: Locations 2, 3 and 4 cause different illumination distribution at QZ

##### 5.3.2.2.2	Rx beam profiles with an independent beam management (IBM) UE

In this sub-clause, an explanation is shown that it is possible to measure the appropriate EIS spherical coverage beam profiles by the test system which equips the non co-located (offset) antenna even with the inter-band 2 DL CA cases as long as the UE is supporting the independent beam management (IBM).

First we consider a single carrier Rx spherical coverage beam profile which we can obtain by two kinds of antennae. If we compare Rx beam profiles of two different cases, one which has been measured by main antenna of the OTA test system, or the other with which the DL beam frequency is same but has been measured from the offset antenna, both beam profiles can be assumed identical as far as following conditions are satisfied.

-	Two measurement antennae (main and offset) are arranged along with the θ rotation of the positioner

-	DL power of the offset antenna is calibrated and capable of transmitting same power level with the main measurement antenna.

For the rotation angles of positioner, refer to Annex D.2.6 in TS 38.810 [3].

Figure 5.3.2.2.2-1 depicts the image of two beam profiles obtained by different antennae. Note that the profiles are obtained one by one since the link has to be maintained with either of antennae during the measurement.

Figure 5.3.2.2.2-1: 2D image of beam profile obtained by two antennae

If we compare the two measurements, a difference between them are just a point of sight from the UE, in other words a boresight of UE is slightly rotated depending on the arrangement of measured antennae.

There is an angular offset between the two measurement antennae such as 2.5 to 7 degrees. Therefore to compare the two obtained beam profiles, rotation of either one of profiles in accordance with the actual antenna alignment is necessary. Also the adjustment of the start/ stop coordinates to measure is necessary with a case of the offset antenna.

Now we consider beam profiles which are obtained by the test system that transmits two DL signals from single antenna (system A), and the system which has one additional offset antenna to transmit two DL signals - one DL from the main antenna and the other from the offset antenna (system B). For inter-band 2 DL CA case for example with band n260 and n261, suppose only system B transmits n261 from the offset antenna, properties of each beam profile are summarized in Table 5.3.2.2.2-1. Note that 2 measurement antennae of system B are assumed to be arranged along with the θ rotation of the positioner again just as mentioned above.

Table 5.3.2.2.2-1: Beam profiles obtained by system A and system B in a case with IBM antennae in a UE

|  | System A (2 DL from main antenna, 1 AoA) | System B (2 DL from slightly offset 2 AoA antennae) |
| --- | --- | --- |
| Beam profile of n260 | Obtained by main antenna | Obtained by main antenna. Thus completely identical with system A. |
| Beam profile of n261 | Obtained by main antenna | Obtained by the offset antenna. The shape of profile should be same but rotated in accordance with the angular offset between two test antennae. |

Taking all explanations above into consideration, system B requires a post processing of the obtained data in accordance with the slightly rotated coordinate system. But the obtained beam profiles can be assumed as identical with ones obtained by system A as far as the UE is supporting the IBM. i.e. There is a way to make IBM UEs to choose same relative beam direction and conduct spherical coverage tests properly like a single test antenna system.

Choice of the beam by UEs with common beam management (CBM) is studied at the next sub-clause.

##### 5.3.2.2.3	Propensity to trigger incorrect beam in CBM UE

The primary mechanism that can mislead CBM UEs is band-specific beam tilt in the test setup. As shown in sub-clause 5.3.2.2.1, beam tilt at the QZ is roughly 1:1 with source angular offset. In the spherical coverage space of a UE, beam tilt becomes a significant problem for beam management at beam boundaries. The impact to measured performance is directly related to probability of finding a beam boundary during 3D search. Beam tilt therefore penalizes UEs with dense beam packing (i.e more beam boundaries).

Dense beam packing is often associated with UEs with good spherical coverage. It can be reasonably argued that the impact of beam tilt in this context is worse for UEs with better spherical coverage. This problem does not have a systematic effect across all UE designs and therefore difficult to overcome.

Another important characteristic of a far field scenario is a nearly planar wavefront. Figures 5.3.2.2.3-1 shows the deviation from planar wavefront at the QZ when the source is moved off-focus. As before, in the figures below, locations 2,3 and 4 have similar angular offset, but differ in mirror to source distance. The figures show that there is an optimal distance from the mirror for any angular offset of the source, for the wavefront to appear planar at the QZ.

Figure 5.3.2.2.3-1: Wavefront orientation at QZ due to off-focus source

Also note that positioning an offset source at the optimal distance may not result in optimal illumination density of the QZ. From a beam management standpoint however, it is more important to ensure quality of the wavefront given that the deviation in illumination density seems minor. (Explanation: Position 3 is close to the optimal point, as witnessed by its nearly flat wavefront in figures 5.3.2.2.3-1. In figure 5.3.2.2.3-1, illumination density curves of location 3 and location 1, the focus or ‘ideal location’, track closely)

The figures 5.3.2.2.3-1 also reasonably predict that the non-linear phase variation varies with frequency. Ergo, it is advantageous to reserve the ideal location (focus) for the antenna serving the highest frequencies and use offset locations for antennae serving lower frequencies.

##### 5.3.2.2.4	Spherical coverage measurement simulation with common beam management (CBM) UE

For inter-band CA requirements within 28 GHz range (L + L) or 39 GHz range (H + H) which has a possibility of transmissions by CBM, a simulation was made on the influence of the offset antenna measurement for inter-band CA case with CBM UE. From this simulation we tried to clarify a difference with the measurement of 2 DL by 1 AoA at a frequency range from 37 GHz and 43.5 GHz.

Assumption of the UE antenna inter-element distance

It is difficult for us to expect an actual inter-element distance (D) of an antenna in the current UE since it is implementation dependent. But as a starting point, during this simulation we put an assumption that an optimization of the inter-element distance is made at the centre frequency between the lower edge of band n258 (24.25 GHz) and higher edge of n259 (43.5 GHz), i.e. 33.875 GHz where D/λ becomes 0.5.

Table 5.3.2.2.4-1: Optimized frequency and corresponding ratio (D/l) for simulation frequencies

| Optimized frequency (D/ = 0.5) | Frequency 1 (f1) for simulation | Frequency 2 (f2) for simulation |
| --- | --- | --- |
| 33.875 GHz (0.5) | 37.0 GHz (0.55) | 43.5 GHz (0.64) |

Assumption of phase shifter in a UE

For a beam forming method of the UE, following two methods were applied in the simulation.

i)	A fixed phase shift to the antenna regardless with the carrier frequencies, which we assume this method is causing the beam squint.

ii)	A different phase shift which is proportional to the carrier frequencies so called true time delay (TTD).

Simulation procedures for spherical coverage

The simulation of the spherical coverage measurement was carried out by the following steps.

Step 1)	Decide a code book to obtain the maximum sensitivity at frequency 1 (37.0 GHz) with a measurement grid placed randomly against a DUT.

Step 2)	Keep the identified code book at step 1) and calculate gain profiles at frequency 2 (43.5 GHz). Then compare the following two cases.

Case 1)	Gain profile at frequency 2 measured by the main antenna

-	Only the difference of the ratio (D/λ) can be monitored as the difference from the frequency 1. This result can be assumed as a baseline when comparing the influence of the offset antenna measurement.

Case 2)	Gain profile at frequency 2 measured by the offset antenna

-	In addition to the difference of the ratio (D/λ), influence of the offset antenna is included in this result. (e.g. influence to the measurement grid due to the offset of the antenna.)

Simulation parameter

Simulation parameters are summarized in Table 5.3.2.2.4-2.

Table 5.3.2.2.4-2: Simulation parameter for spherical coverage gain profile

| Item | Parameter |
| --- | --- |
| Grid | Constant Density 200 pt, 1000 random orientations against a DUT. |
| Antenna offset |  = 4.0 degrees |
| Array antenna | 1 x 4. Inter-element distance is optimized at 33.875 GHz. |
| Phase shifter | Non TTD (Constant Phase), TTD (True Time Delay) |
| Carrier frequency | 37.0 GHz, 43.5 GHz |

Spherical coverage simulation result

Simulation results of the spherical coverage are summarized in Table 5.3.2.2.4-3 and 5.3.2.2.4-4.

Table 5.3.2.2.4-3: Simulation result of spherical coverage – Mean 50%-ile error (against 1deg uniform grid as a reference)

| D/ Test frequency) | Non-TTD |  | TTD |  |
| --- | --- | --- | --- | --- |
|  | Main antenna | Offset antenna | Main antenna | Offset antenna |
| 0.55 (37.0 GHz) | 0.024 | - | Same as Non-TTD | - |
| 0.64 (43.5 GHz) | 0.016 | 0.101 | 0.019 | 0.135 |

Table 5.3.2.2.4-4: Simulation result of spherical coverage – Standard deviation of 50%-ile value

| D/ Test frequency) | Non-TTD |  | TTD |  |
| --- | --- | --- | --- | --- |
|  | Main antenna | Offset antenna | Main antenna | Offset antenna |
| 0.55 (37.0 GHz) | 0.059 | - | Same as Non-TTD | - |
| 0.64 (43.5 GHz) | 0.096 | 0.102 | 0.111 | 0.168 |

Comparing the results between the main antenna and offset antenna, we observed that the mean error by measuring from the offset antenna increases slightly with both non-TTD and TTD type phase shifter (0.085 dB with non-TTD and 0.116 dB with TTD at 43.5 GHz.). This means that the total measurement uncertainty by measuring from offset antenna will increase approximately 0.1 dB as the systematic uncertainty.

For the standard deviation, only the result with TTD phase shifter showed the slight increase of measurement uncertainty (0.057 at 43.5 GHz.). However this increase of random uncertainty should be a negligible level compared to the other uncertainty contribution.

Therefore for both mean error (systematic error) and standard deviation (random error) with offset antenna, these values are within the acceptable level because the simulation assumptions in this paper are chosen as one of the severest conditions from the viewpoint of the frequency point to optimize inter-element distance, test frequencies, basic frequency point of phase shift, etc. And since the optimization of the inter-element distance was assumed at 33.875 GHz, if this optimization is made at a higher frequency such as at the middle of 37 GHz and 43.5 GHz, the observed uncertainty should be decreased.

Considering all the observations above, we assume that the FR2 OTA test system with the offset test antenna has a feasibility of measuring inter-band DL CA test cases for both CBM and IBM UEs under some limitations.

For cases with UEs which supports wider frequencies (such as n262 in addition) or higher power such as PC1 need a further study.

#### 5.3.2.3	Summary on applicability of offset antenna test system

Taking into consideration of study results in sub-clause 5.3.2.1 and 5.3.2.2, an applicability of the offset antenna test system to the spherical coverage test with an inter-band CA UE is summarized as follows.

IBM UE:

On a test for UEs supporting inter-band CA with IBM, there is a way to make IBM UEs to choose same relative beam direction and conduct spherical coverage tests properly like a single test antenna system. It is recommended that a design of the test system is optimized to mitigate an impact of the offset antenna to the QoQZ measurement uncertainty, including the diffraction and/or scattering effects created by the reflector’s paraboloid edges and the size of the paraboloid.

CBM UE:

On a test for UEs supporting inter-band CA with CBM, there might be a limitation with the feasibility by the offset antenna test system. But this relates also to the on-going WI discussion on the necessity of spherical coverage requirements with CBM UEs supporting a same band group e.g. 28 GHz + 28 GHz.

#### 5.3.2.4	Points to design the FR2 OTA test system with offset test antennae

To design FR2 OTA IFF test system for inter-band CA UE, following points were analyzed in the previous sub-clauses with regards to an impact of off-focus test antennae.

-	Distance (and angular offset) between the main antenna and the offset antenna

-	Arrangement of the offset antenna

-	Angle to fix (tilt) the offset antenna

-	Distance between the offset antenna and a reflector

-	Arrangement of antennae associated with their frequency coverage

We pick out and iterate examples from the previous analyses which need to be considered during a design of the test system to minimize impacts from the offset antenna. We also consider further with their feasibility from some other aspects e.g. another affecting factor, actual system assembly procedures and test operations.

Distance (and angular offset) between main antenna and offset antenna

Based on the assumption that a range length in an IFF chamber is from 800 mm to 1200 mm, antenna offset from 50 mm to 100 mm were studied. These offsets are equivalent to approximately 2.5 to 7 degrees as an angular offset. And due to a shift of focal point from a centre of the quiet zone, approximately 0.15 dB degradation of the quality of quiet zone MU is estimated at the frequency range from 23.45 to 40.8 GHz. Note the estimation may vary depending on components and system designs in each vendor.

From the previous studies, it is obvious that the impact is proportional to the distance between antennae. From this point it is preferrable that each antenna is arranged as close as possible. However another effect of mutual coupling arises if the gap becomes too close like that of one wavelength (i.e. approximately 10 mm around 30 GHz.). Though we can assume that the effect can be included while evaluating the quality of quiet zone, at the same time when we think of an assembly of cables to each antenna, some clearances must be ensured and thus a gap around 40 to 50 mm between antennae is considered possible closest distance.

Arrangement of the offset antenna

On condition that a DUT is supporting IBM, we have a chance to carry out the inter-band CA test case with the offset antenna test system equivalent to the system only with the main antenna. To obtain identical beam profiles from both of main and offset antenna, following conditions need to be satisfied:

-	Two measurement antennae (main and offset) are arranged along with the θ rotation of the positioner

-	DL power of the offset antenna is calibrated and capable of transmitting same power level with the main measurement antenna.

Since there is an angular offset between the two measurement antennae such as 2.5 to 7 degrees, to compare the two obtained beam profiles, a post processing of obtained data is necessary with either one of profiles in accordance with the actual antenna alignment. Also the adjustment of the start/ stop coordinates to measure is necessary with the offset antenna.

Angle to fix (tilt) the offset antenna

It is recommended that also an angle to fix (tilt) the offset antenna is considered while designing the test system. It is possible to shift the electric field intensity by tilting the offset antenna (e.g. 0.5 to 0.7 degree) and make the distribution of the field intensity, in other words an amplitude taper, close to symmetric in the quiet zone like the one from the main antenna. However since the applicable tilt angles are closely related with a size of reflector and the range length, there should be some limitations and care must be taken to avoid diffraction and/or scattering effects created by the reflector’s paraboloid edges.

Distance between the offset antenna and a reflector

It is possible to optimize the shape of wavefront and illumination at the centre of the quiet zone by adjusting a distance between the offset antenna and a reflector. However since the QZ illumination also depends on the frequency of beam, and considering the current frequency range of mmWave region, which is wide spread from 24 GHz to 52 GHz, it is not practical to change the distance for each antenna one by one. Also when we consider a design that the placement of the offset antenna is above the main antenna, there might be another factor to increase the measurement uncertainty since coordinates of the measurement grid changes. Thus as mentioned above, it is recommended that antennas are arranged along with the θ rotation of the positioner

Arrangement of antennae associated with their frequency coverage

It is predicted that the non-linear phase variation varies with frequency. Ergo, it is advantageous to reserve the ideal location (focus) for the antenna serving the highest frequencies and use offset locations for antennae serving lower frequencies.

### 5.3.3 	Inter-band testing ramifications

The primary dependency of inter-band test set ramifications is the frequency coverage of each antenna in an IFF system with multiple antennae.

#### 5.3.3.1 	Single antenna

For bands that can be supported by a single antenna, the condition to evaluate is if the antenna is not at the focus of the mirror. On-focus single antenna IFF systems have already been studied and their MU quantified, and do not need to be considered again in this context. Non-ideal (off-focus) location causes the following problems:

-	A shifted QZ due to beam tilt. Note however that all bands have the same AoA at the UE.

-	Beam tilt causes the AoA to no longer be parallel to the optical axis. This aspect must be properly considered during system calibration; for example, a directional calibration antenna placed at the QZ must be pointed accurately along the arrival direction for an accurate path loss estimate.

Provided the problems above are accounted for, UEs with either CBM or IBM can tolerate an IFF system with an off-focus source.

#### 5.3.3.2 	Multiple antennae

The assumption for inter-band testing in this context is that the bands are supported through multiple non-co-located test system antennae. For this set up, the considerations listed in the single antenna case get further complicated.

-	A reduction in QZ size due to different beam tilt experienced by different bands.

-	Beam tilt causes different AoA for different bands. There are two problems associated with this aspect:

-	Calibration for each band will require adjustment of a directional calibration antenna so it is pointed along arrival direction of band being calibrated.

-	This set up can be perceived as ‘non-co-located’ gNBs by some UEs.

The calibration step complication, and the QZ size reduction may be surmountable, but the non-co-located gNB implication can cause significant problems for UEs with CBM limitation.

A special note is warranted for ACS and IBB requirements – the standard requires that the interferer share the same AoA as the DL band being tested. Fortunately, ACS and IBB interferers are expected to be in the same band as the DL being tested, so it would be natural for the test equipment to use the same antenna for both. If such is not the case, additional MU would be introduced into the system, which is not preferred.

## 5.4	Extreme temperature conditions

### 5.4.1	ETC test system

Permitted test methods (i.e. DFF, IFF, NFTF) defined in Clause 5, TR 38.810 [3] can support extreme temperature condition tests with the update of additional temperature control system. An example of IFF-based ETC test system is shown in Figure 5.4.1-1 below.

Figure 5.4.1-1: An example of an IFF-based ETC test system

The key aspects of the ETC setup are:

-	The test system should support the temperature range for extreme conditions, i.e. -10C to +55C, defined in Annex E.2.1 in TS 38.101-2 [2].

-	The criterion temperature tolerance is defined in 5.4.5

-	A positioning system can support 3D scan.

### 5.4.2	Calibration procedure

The path loss calibration should be performed with the ETC enclosure surrounding the calibration reference antenna. All measurements performed with the ETC enclosure in place shall use the path loss calibration performed under this condition.

### 5.4.3	Test procedure

For EIRP/EIS beam peak searching procedure under ETC, two test procedures are available:

-	Option 1: perform 3D scan

-	Option 2: beam peak search within a certain cone of directions around peak position under NTC (by declaration or NTC peak searching results)

By default, 3D scan is used for ETC tests. If a certain cone of directions around peak position under NTC can be declared by UE vendor or be got from NTC peak searching results, then option 2 can be used.

Note: 3D scan (option 1) is needed for UE with best antenna panel switched by temperature variation and/or UE without declaration present.

### 5.4.4	Temperature tolerance limit of ETC test system

The temperature tolerance for FR2 ETC system should be defined, and the test can only be executed under target temperature within the tolerance. At least two aspects need to consider:

-	An accuracy of temperature control by an air conditioner

-	Accuracy of a thermocouple to measure a temperature in the ETC enclosure

The recommended temperature tolerance limit of FR2 ETC system is ±[4]ºC.

## 5.5	Extension of frequency applicability for band n262

Editor’s note: outcome of SI Objective 6 related to the extension of frequency applicability of the permitted methods is captured in this clause

## 5.6	Extension of frequency applicability for FR2-2

### 5.6.1	Permitted test methods

Unless otherwise stated, FR2-2 will follow the baseline UE RF methodology detailed in TR 38.810. This includes the extension of frequency applicability of permitted test methods in Clause 5.2 of TR 38.810 and the general testing and calibration aspects captured in Clause 5.2.1.3.

### 5.6.2	Enhanced test methods

#### 5.6.2.1	High DL power and low UL power

#### 5.6.2.2	Polarization basis mismatch between the TE and DUT

Unless otherwise stated, applicability of the polarization basis mismatch solutions found in Clause 5.2 of this TR, shall be extended to FR2-2.

#### 5.6.2.3	Inter-band (FR2+FR2) CA

#### 5.6.2.4	Extreme temperature conditions

#### 5.6.2.5	Test time reduction

Unless otherwise stated, applicability of the enhanced test methods defined in Clause 8.2 (RSRP(B) based RX beam peak search), Clause 8.4 (Single link polarization measurement) and Clause 8.5 (Other methods) of this TR, shall be extended to FR2-2.

# 6	UE RRM testing methodology enhancements

## 6.1	Extension of frequency applicability for band n262

Following the derivation of band-dependent parameters provided in Clause 7.1 and reusing the assumptions from Clause 6.2.1.4 of TR38.810 related to AoA scenarios and requirement types, the maximum achievable SNR for the RRM test setups with a PC3 DUT can be summarized as shown in Table 6.1-1 below.

Table 6.1-1: Comparison of maximum SNR between TR38.810 and preliminary extension to band n262

| RRM test setup | UE | CBW (MHz) | Max SNR (dB) [TR38.810] | Max SNR (dB) [n262] |
| --- | --- | --- | --- | --- |
| Scenario 1 (1AoA RX beam peak) for Type 1 Requirements (“Fine” RX beams) and Mode 1 Configuration (S+N) | Single band UE | 100 | [19.7] | [15.2] |
|  |  | 200 | [16.7] | [12.2] |
|  | Multi-band UE (NOTEs 1,2) | 100 | [17.7] | [14.2] |
|  |  | 200 | [14.6] | [11.2] |
| Scenario 1 (1AoA RX beam peak) for Type 2 Requirements (“Rough” RX beams) and Mode 1 Configuration (S+N) | Single band UE | 100 | [12.5] | [7.6] |
|  |  | 200 | [9.5] | [4.6] |
|  | Multi-band UE (NOTEs 1,2) | 100 | [10.5] | [6.6] |
|  |  | 200 | [7.5] | [3.6] |
| Scenario 2 (1AoA RX non-beam peak) for Type 1 Requirements (“Fine” RX beams) and Mode 1 Configuration (S+N) | Single band UE | 100 | [6.7] | [-2.7] |
|  |  | 200 | [3.7] | [-5.7] |
|  | Multi-band UE (NOTEs 1,2) | 100 | [4.7] | [-3.7] |
|  |  | 200 | [1.7] | [-6.7] |
| Scenario 2 (1AoA RX non-beam peak) for Type 2 Requirements (“Rough” RX beams) and Mode 1 Configuration (S+N) | Single band UE | 100 | Not usable | Not usable |
|  |  | 200 | Not usable | Not usable |
|  | Multi-band UE (NOTEs 1,2) | 100 | Not usable | Not usable |
|  |  | 200 | Not usable | Not usable |
| NOTE 1: For ∑MBp from TS 38.101-2 [2] Table 6.2.1.3-4 allows up to 2 dB in Rel-15 NOTE 2: From Rel-16 and later ∑MBp can be rounded up to 1.0 dB for all bands NOTE 3: The parameters and values in this table are preliminary and subject to further refinement by RAN5 as part of their conformance test development work. |  |  |  |  |

## 6.2	Extension of frequency applicability for FR2-2

### 6.2.1	General

Unless otherwise stated, all test methods and measurement setup for FR2 RRM methodology defined in TR 38.810 [3] Clause 6 and in TS 38.508-1 [10] Clause 7 are applicable for FR2-2.

### 6.2.2	Noc methodology

The Noc level for Scenario 1 (1AoA RX beam peak) for Type 1 Requirements ("Fine" RX beams) and Mode 1 Configuration (S+N) described in TR 38.810 Clause 6.2.1.4.3 need to be adjusted and can be derived similar to the Noc level for UE demodulation test methods described in Clause 7.2.2.

### 6.2.3	Maximum SNR derivation

Following the derivation of band-dependent parameters provided in Clause 7.2 and reusing the assumptions from Clause 6.2.1.4 of TR 38.810 related to AoA scenarios and requirement types, the maximum achievable SNR for the RRM test setups can be summarized as shown in Table 6.2.3-1.

Table 6.2.3-1: Maximum SNR preliminary extension to band n263

| RRM test setup | UE | CBW (MHz) | Max SNR (dB) [n263] |
| --- | --- | --- | --- |
| Scenario 1 (1AoA RX beam peak) for Type 1 Requirements (“Fine” RX beams) and Mode 1 Configuration (S+N) | Single band UE | 100 | [7.7] |
|  |  | 400 | [-0.6] |
|  | Multi-band UE | 100 | TBD |
|  |  | 400 | TBD |
| Scenario 1 (1AoA RX beam peak) for Type 2 Requirements (“Rough” RX beams) and Mode 1 Configuration (S+N) | Single band UE | 100 | < -20 (NOTE 1) |
|  |  | 400 | < -20 (NOTE 1) |
|  | Multi-band UE | 100 | TBD |
|  |  | 400 | TBD |
| Scenario 2 (1AoA RX non-beam peak) for Type 1 Requirements (“Fine” RX beams) and Mode 1 Configuration (S+N) | Single band UE | 100 | < -20 (NOTE 1) |
|  |  | 400 | < -20 (NOTE 1) |
|  | Multi-band UE | 100 | TBD |
|  |  | 400 | TBD |
| Scenario 2 (1AoA RX non-beam peak) for Type 2 Requirements (“Rough” RX beams) and Mode 1 Configuration (S+N) | Single band UE | 100 | < -20 (NOTE 1) |
|  |  | 400 | < -20 (NOTE 1) |
|  | Multi-band UE | 100 | TBD |
|  |  | 400 | TBD |
| NOTE 1: Result does not converge |  |  |  |

# 7	UE demodulation testing methodology enhancements

Editor’s note: testing and calibration aspects of the permitted methods for FR2 UE demodulation testing and the preliminary assessment of measurement uncertainty (Clause 7.2 and Annex B.2 of TR38.810) define the baseline UE demodulation methodology for the purpose of this study.

## 7.1	Extension of frequency applicability for band n262

Annex B.3 in TR38.810 describes the detailed calculations of the maximum achievable SNR by the demodulation test setup.  Although range length for the DFF setup is not a requirement for permitted methods, a reference value of 0.725m is used in the TR38.810 calculations.  Furthermore, the maximum achievable SNR values are calculated assuming the highest band n259 frequency (f=43.5 GHz).  The highest frequency to be considered for the SNR calculation is the highest frequency of band n262:  48.2 GHz.  Using the reference range length of 0.725m we determine free space path loss at f=48.2 GHz, as shown in Figure 7.1-1 below.

Figure 7.1-1: Free space path loss vs. frequency for d=0.725m

Based on this analysis, the relative increase in free space path loss from 43.5 GHz to 48.2 GHz is 0.9 dB.

Another parameter in the SNR calculation which scales with frequency is cable loss per meter.  Using the datasheets from five different commercially available RF cables, which are rated up to 50 GHz, the average cable loss per meter is calculated (averaging performed in linear gain units), as shown in Figure 7.1-2 below.

Figure 7.1-2: Cable loss per meter vs. frequency

Based on this analysis, the relative increase in cable loss per meter from 43.5 GHz to 48.2 GHz is 0.33 dB.

The SNR calculation related to cable loss also includes connector losses and additional margin, and it is feasible to reuse these parameters for the calculations related to band n262.

The SNR calculation also includes two parameters which are taken from UE RF requirements:  REFSENS and multi-band relaxation.  For REFSENS the recently agreed value of -82.8 dBm/50 MHz is used, and for MBR 1.0 dB (rounded up from 0.7 dB) is used [2].  Table 7.1-1 below summarizes the parameters.

Table 7.1-1: Preliminary demodulation test setup SNR calculation parameters for band n262

| Parameter | Value | Comment |
| --- | --- | --- |
| REFSENS | -82.8 dBm/50 MHz | Using REFSENS agreed for band n262 |
| Multi-band relaxation | 1.0 dB | Defined as ceil(.); change from 2.0 dB |
| FS path loss | -63.2 dB | Change from -62.3 dB (scaling from 43.5 to 48.2 GHz) |
| Cable loss | -8.7 dB | Additional 0.33 dB/m in cable loss at 48.2 GHz |
| Probe antenna gain | [12.0] dB |  |
| Backoff from P1dB | [13.0] dB |  |
| NOTE: The parameters and values in this table are preliminary and subject to further refinement by RAN5 as part of their conformance test development work. |  |  |

The maximum achievable SNR for the demodulation test setup can be summarized as shown in Table 7.1-2.

Table 7.1-2: Comparison of maximum SNR between TR38.810 and preliminary extension to band n262

|  | CBW (MHz) | Max SNR (TR38.810) | Max SNR (n262) |
| --- | --- | --- | --- |
| Single band UE | 100 | [19.7 dB] | [15.2] |
|  | 200 | [16.7 dB] | [12.2] |
| Multi-band UE (NOTEs 1,2) | 100 | [17.7 dB] | [14.2] |
|  | 200 | [14.6 dB] | [11.2] |
| NOTE 1: For ∑MBp from TS 38.101-2 [2] Table 6.2.1.3-4 allows up to 2 dB in Rel-15 NOTE 2: From Rel-16 and later ∑MBp can be rounded up to 1.0 dB for all bands |  |  |  |

In general, there is a 3.5 dB degradation in maximum achievable SNR for band n262 relative to the budgeted values in TR38.810.

## 7.2	Extension of frequency applicability for FR2-2

### 7.2.1	General

Unless otherwise stated, all test methods and measurement setup for FR2 UE demodulation and CSI testing methodology defined in TR 38.810 Clause 7 [3] are applicable for FR2-2.

### 7.2.2	Noc methodology

The following methodology to define the minimum Noc level for power class X (PC_X) and operating band Y (Band_Y) is used for the single carrier case and a single band device:

-	Noc (PC_X, Band_Y) = RESFENSPCX, BandY -10log10(SCSREFSENS x PRBREFSENS x 12) - SNRREFSENS + ∆thermal

-	REFSENSPCX, BandY is the REFSENS value in dBm specified for Power Class X UE in band Y for [100MHz] channel bandwidth

-	SCSREFSENS is [120 kHz].

-	PRBREFSENS is NRB associated with subcarrier spacing SCSREFSENS for [100MHz] channel bandwidth

-	12 is the number of subcarriers in a PRB

-	SNRREFSENS = -1 dB is the SNR used for simulation of REFSENS

-	∆thermal is the amount of dB that the wanted noise is set above UE thermal noise, giving a rise in total noise of ∆BB. ∆thermal = 6dB, giving a rise in total noise of 1dB.

Note: Further confirmation of used parameters is needed based on core requirements definition.

### 7.2.3	Maximum SNR derivation

This clause includes an informational assessment of testable DL SNR range for FR2-2 for maximum frequency (~71GHz) using TR38.810 methodology. Annex B.3 in TR38.810 describes the detailed calculations of the maximum achievable SNR by the demodulation test setup.  As an informative assessment, calculation of the maximum testable DL SNR for band n263 is performed below. IFF test method is considered.

Although range length for the DFF setup is not a requirement for permitted methods, a reference value of 0.725m is used in the TR38.810 calculationsThe highest frequency to be considered for the SNR calculation is the highest frequency of band n263:  f=71 GHz.  Using the reference range length of 0.725m we determine free space path loss, together with modeling of atmospheric absorption, at f=71 GHz, as shown in Figure 7.2.3-1 below.

Figure 7.2.3-1: Free space path loss vs. frequency for d=0.725m

We note that atmospheric attenuation has no impact on path loss at the range scales of an FR2 testing chamber.

Another parameter in the SNR calculation which scales with frequency is cable loss per meter.  Using the datasheets from four different commercially available RF cables, which are rated up to at least 90 GHz, we calculate the average cable loss per meter (averaging performed in linear gain units), as shown in Figure 7.2.3-2 below.

Figure 7.2.3-2: Cable loss per meter vs. frequency

We further note that the SNR calculation related to cable loss also includes connector losses and additional margin.Demodulation test system setup parameters required for SNR calculation are summarized in Table 7.2.3-1.

Table 7.2.3-1: Demodulation test setup SNR calculation parameters for band n263

| Parameter | Comment |
| --- | --- |
| REFSENS | Using REFSENS agreed for band n263 -72.0 dBm/400 MHz |
| Multi-band relaxation | 23 dBm |
| TE amplifier 1dB compression | TBD |
| Backoff from P1dB | -13 dB (RAN4 assumption) -17.7 dB (RAN5 assumption) |
| Cable loss | 10.3 dB, assuming 1m length |
| Connector insertion loss | 0 |
| FS path loss | 66.7 dB, assuming 0.725m range length |
| TE DL absolute power setting uncertainty | +/-6 dB |
| Probe antenna gain | 12 dBi |
| Beam peak search procedure error | 0.5 dB |

The maximum achievable DL SNR is summarized in Table 7.2.3-2.

Table 7.2.3-2: Maximum DL testable SNR preliminary extension for band n263

|  | CBW (MHz) | Test method |
| --- | --- | --- |
|  |  | IFF |
| Single band UE | 100 | [7.7] |
|  | 400 | [-0.6] |
|  | 800 | [-14.5] |
|  | 1600 | < -20 (NOTE 1) |
|  | 2000 | < -20 (NOTE 1) |
| Multi band UE | 100 | TBD |
|  | 400 | TBD |
|  | 800 | TBD |
|  | 1600 | TBD |
|  | 2000 | TBD |
| NOTE 1: Result does not converge |  |  |

A method to adjust ∆thermal value in Noc setting methodology can increase max achievable DL SNR. ∆thermal is the actual difference between UE thermal noise level and wanted noise level. The difference between these levels leads to the difference between baseband SNR and SNR at the reference point. According to the FR2-1 SNR calculation methodology, 6 dB ∆thermal is used that corresponds to 1 dB SNR difference.

Table 7.2.3-2 below provides evaluations how ∆thermal impacts DL SNR. For simplicity the ∆thermal value is recalculated to allowed SNR difference (allowed noise increase). 400 MHz CBW and -13 dB backoff from P1dB were selected for the analysis. Note that at 3 dB allowed noise increase, Noc level becomes the same as UE thermal noise level.

Table 7.2.3-3: Allowed noise increase vs max achievable DL SNR

| Allowed noise increase, dB | 1 | 2 | 3 |
| --- | --- | --- | --- |
| ∆thermal, dBm/Hz | 6.1 | 2.6 | 0.1 |
| Max DL SNR BB, dB | 1.1 | 5 | 6.8 |

Minimum FR2-1 DL radiated performance requirements are specified for up to Rank 2. The required SNR point is 14.4 and 18.6 dB for 16QAM and 64QAM respectively for 100 MHz and 120 kHz CBW/SCS combination. It can be approximately expected that SNR will not dramatically change for higher CBWs as 400MHz. Same time, some margin on phase noise impact at higher carrier frequency should be considered. For instance, 16 and 20 dB SNRs can be assumed. Such SNR targets require adjustments of TE parameters. Table 7.2.3-4 summarizes required wanted signal + headroom budget to reach 16 and 20 dB DL SNR. The parameters from Table 7.2.3-1 are assumed with RAN4 assumption on backoff from P1dB.

Table 7.2.3-4: Wanted signal + headroom budget vs max achievable DL SNR

| Target DL SNR, dB | 16 | 20 |
| --- | --- | --- |
| wanted signal + headroom, dBm/ Ch BW | 35.8 | 39.8 |

Wanted signal + headroom budget can be increased by adjustment of power amplifier 1dB compression point and/or probe antenna gain. Another option how to allow testing of high CBW is to restrict max PRB allocation size within the CBW. As seen from Table 7.2.3-2, 66 PRB allocation size (100 MHz) leads to 7.7 dB max SNR increase compared to 264PRBs in 400 MHz.

# 8	Test time reduction

## 8.1	General

The verification methodologies for FR2 UE RF, UE RRM, and UE demodulation requirements are all OTA measurements. Given the complexity of OTA test system, the test time of RF, RRM and demodulation test have been dramatically increased compare with FR1 conducted test cases.

An example of test time of some FR2 RF test cases is listed in Table 8.1-1.

Table 8.1-1: Feedback of actual FR2 testing time from one TE vendor (example)

| FR2 test cases based on TS38.521-3/2: |  |  | Time/h or min |
| --- | --- | --- | --- |
| 38.521-2 |  | Tx beam peak direction search | 4h (with 7.5° step) |
| 38.521-2 |  | Rx beam peak direction search | 11h (with 7.5° step) |
| 38.521-3 | 6.2B.1.4.1 | UE Maximum Output Power for Inter-Band EN-DC including FR2 (2 CCs) - EIRP and TRP | 30min |
| 38.521-3 | 6.2B.1.4.2 | UE Maximum Output Power for Inter-Band EN-DC including FR2 (2 CCs) - Spherical Coverage | 1h |
| 38.521-3 | 6.3B.2.4 | Transmit OFF Power for inter-band EN-DC including FR2 | 15min |
| 38.521-3 | 6.5B.2.4.1 | Spectrum emissions mask for Inter-band EN-DC including FR2 (2 CCs) | 35min |
| 38.521-3 | 6.5B.2.4.3 | Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (2 CCs) | 35min |
| 38.521-3 | 6.5B.3.4.1 | General Spurious Emissions for Inter-band including FR2 (2 CCs) | 1h |
| Note: The above testing time is varied due to different UE performance, Test software version, and detailed parameters setting. |  |  |  |

In addition, given all the FR2 UE should be tested with the DUT operated in stand-alone battery powered mode [3], which means much power charging time should also be considered, then the total testing time would be further increased. Therefore, proper approaches to reduce the FR2 test time significantly is a key issue to be resolved.

## 8.2	New measurement grid

### 8.2.1	New measurement grids based on 4x2 antenna pattern assumption

For PC3 UEs, an 4x2 antenna array has been agreed for measurement grid analyses. The Table 8.2.1-1 and Table 8.2.1-2 outline the antenna patterns for simulation.

Table 8.2.1-1: Single Antenna Element Radiation Pattern

| Antenna element horizontal radiation pattern | AE,Hφ=-min12φφ3dB2,Am, Am = 30 dB |
| --- | --- |
| Horizontal half-power beamwidth of single element | 260° |
| Antenna element vertical radiation pattern | AE,Vφ=-min12θ-90θ3dB2,SLAV, SLAV = 30 dB |
| Vertical half-power beamwidth of single array element | 130º |
| Array element radiation pattern | AEφ,θ=GE,max-minAE,Hφ+AE,Vθ,Am, GE,max = 1.5 dBi |
| Element gain without antenna losses | GE,max = 1.5 dBi |

Table 8.2.1-2: Composite Antenna Array Radiation Pattern

| Composite array radiation pattern in dB AEφ,θ | AA,Beamsθ,φ=AEφ,θ+10log10m=1NHn=1NVwi,n,m∙vn,m2 the super position vector is given by: vn,m=expi∙2πn-1∙dVλ∙cosθ+m-1∙dHλ∙sinθ∙sinφ, n=1,2,…NV;m=1,2,…NH; the weighting is given by: wi,n,m=1NHNVexpi∙2πn-1∙dVλ∙sinθi,etilt-m-1∙dHλ∙cosθi,etilt∙sinφi,etilt |
| --- | --- |
| Antenna array configuration (Row×Column) | 4 × 2 |
| Horizontal radiating element spacing dh/λ | 0.5 |
| Vertical radiating element spacing dv/λ | 0.5 |

Based on 4x2 antenna array, the following three types of measurement grids need to be derived:

-	Beam Peak Search Grid: using this grid, the TX and RX beam peak direction will be determined. 3D EIRP scans are used to determine the TX beam peak direction and 3D Throughput/RSRP/EIS scans for RX beam peak directions.

-	Spherical Coverage Grid: using this grid, the CDF of the EIRP/EIS distribution in 3D is calculated to determine the spherical coverage performance.

-	TRP Measurement Grid: using this grid, the total power radiated by the DUT in the TX beam peak direction is determined by integrating the EIRP measurements taken on the sampling grid.

#### 8.2.1.1	Beam Peak Search Measurement Grid

Following the analysis approach in TR38.810 Annex G, similar analyses based on 50k simulations have been performed for the 4x2 antenna array assumption. The global beam peak of the 4x2 antenna array was determined first. Subsequently, the relative orientation of the simulated antenna array and the measurement grid was altered randomly.

Sample histograms and CDF distributions for the beam peak error for constant step-size measurement grids are shown in Figure 8.2.1.1-1 and for the constant density measurement grid (based on the charged particle implementation) in Figure 8.2.1.1-2. The histograms show a half-normal distribution.

Given the half-normal distribution, the MU term should be based on the determination of the offset from the beam peak that contains 95% of the distribution (alternatively, the value at which the CDF is 5%). This offset shall be considered a systematic error in the MU budget. The various statistical metrics are illustrated in Figure 8.2.1.1-3.

Figure 8.2.1.1-1: Histogram of maximum beam peak errors for sample constant-step size meausurement grids (left: 12o, right: 15o step size) for 260o/130o HPBW

Figure 8.2.1.1-2: Histogram of maximum beam peak errors for sample constant density measurement grids (left: 320, right: 200 grid points) for 260o/130o HPBW

Figure 8.2.1.1-3: Statistical metrics for a sample half-normal distribution

The statistical results from simulations using 50k random orientations are then used for further analyses, summarized in Table 8.2.1.1-1 for constant-step size grids and in Table 8.2.1.1-2 for constant-density grids. The simulation assumptions of the rotations were the same as those outlined in Annex G.1.1 of [3]. it should be noted that these measurement grids are derived without consideration of UE beam steering effect (i.e. beam correspondence).

Table 8.2.1.1-1: Statistical Analyses of the 50k simulations for the constant-step size grids

| Angular Step Size [o] | Number of unique grid points | Mean Error [dB] | STD [dB] | Offset5%CDF [dB] |
| --- | --- | --- | --- | --- |
| 7.5 | 1106 | 0.07 | 0.05 | 0.17 |
| 9.0 | 762 | 0.10 | 0.07 | 0.25 |
| 10.0 | 614 | 0.12 | 0.09 | 0.31 |
| 11.25 | 482 | 0.15 | 0.11 | 0.38 |
| 12.0 | 422 | 0.17 | 0.13 | 0.44 |
| 12.86 | 366 | 0.20 | 0.15 | 0.50 |
| 13.8 | 314 | 0.23 | 0.17 | 0.58 |
| 15.0 | 266 | 0.27 | 0.21 | 0.69 |

Table 8.2.1.1-2: Statistical Analyses of the 50k simulations for the constant-density grids

| Number of unique grid points | Mean Error [dB] | STD [dB] | Offset5%CDF [dB] |
| --- | --- | --- | --- |
| 800 | 0.07 | 0.05 | 0.17 |
| 700 | 0.09 | 0.06 | 0.20 |
| 650 | 0.09 | 0.06 | 0.21 |
| 600 | 0.10 | 0.07 | 0.23 |
| 550 | 0.11 | 0.07 | 0.25 |
| 500 | 0.12 | 0.08 | 0.28 |
| 450 | 0.13 | 0.09 | 0.31 |
| 400 | 0.15 | 0.10 | 0.35 |
| 350 | 0.17 | 0.12 | 0.39 |
| 300 | 0.20 | 0.14 | 0.46 |
| 275 | 0.22 | 0.15 | 0.50 |
| 250 | 0.24 | 0.16 | 0.55 |

Based on the previously agreed limit of Offset5%CDF of 0.5dB (systematic error), the following minimum number of grid points would be required for Beam Peak Search Grid.

-	Constant density grid with at least 275 grid points

-	Constant step size grid with at least 366 grid points

Table 8.2.1.1-3: Min Number of Grid Points for TX/RX Beam Peak Search

| Antenna Assumption Grid Type | 8x2 | 4x2 | Factor of Improvement |
| --- | --- | --- | --- |
| Constant-Step Size | 1106 | 366 | 3.0 |
| Constant-Density | 800 | 275 | 2.9 |

The approximate test times for the 4x2 beam peak searches are as follows:

-	Constant-Step Size: TX ~0.7hrs; RX ~4hrs

-	Constant Density: TX ~0.5hrs; RX ~3hrs

#### 8.2.1.2	Spherical Coverage Measurement Grid

The simulation assumptions including the antenna patterns for the spherical coverage measurement grids are the same as Clause G.3 [3] except the 4x2 antenna array assumptions instead of 8x2.

At the 50%-tile CDF, i.e., the target CDF for Power Class 3, statistical analyses of all 10000 EIRPs, EIRP50%CDF, are performed.

The simulations in this contribution were only for the case where the beam peak is oriented in completely random orientations, i.e., the beam peak is not always aligned to a grid point. It is understood that the CDF curve cannot be used to accurately determine the TX beam peak (100%-tile CDF)

Unlike in [3], the simulations here were performed for EIRP only it was shown previously that the EIS simulations with infinitesimal DL power step sizes match the standard deviations of the EIRP results and that a finite DL power step size introduces a mean error that matches the DL power step size.

The results for various constant-step size measurement grids are tabulated in Table 8.2.1.2-1 and the grid with similar MUs as previously agreed for the 8x2 based PC3 configuration is highlighted.

Table 8.2.1.2-1: Statistical results of EIRP50%CDF for the 4x2 antenna array for constant step size measurement grids and the beam peak oriented in completely random orientations.

| Step Size [o] | Number of unique grid points | Std. Dev [dB] | ¦Mean Error¦ [dB] |
| --- | --- | --- | --- |
| 10.0 | 614 | 0.03 | 0.00 |
| 12.0 | 422 | 0.08 | 0.01 |
| 15.0 | 266 | 0.06 | 0.02 |
| 20.0 | 146 | 0.10 | 0.03 |
| 22.5 | 114 | 0.22 | 0.02 |
| 30.0 | 62 | 0.21 | 0.04 |
| 45.0 | 26 | 0.30 | 0.14 |

Similar results for the constant-density measurement grids are tabulated in Table 8.2.1.2-2 and the grid with similar MUs as previously agreed for the 8x2 based PC3 configuration is highlighted.

Table 8.2.1.2-2: Statistical results of EIRP50%CDF for the 4x2 antenna array for constant density measurement grids and the beam peak oriented in completely random orientations.

| Number of unique grid points | Std. Dev [dB] | ¦Mean Error¦ [dB] |
| --- | --- | --- |
| 50 | 0.19 | 0.05 |
| 60 | 0.25 | 0.03 |
| 70 | 0.21 | 0.04 |
| 80 | 0.22 | 0.03 |
| 90 | 0.14 | 0.03 |
| 100 | 0.12 | 0.03 |
| 110 | 0.10 | 0.03 |
| 120 | 0.09 | 0.03 |
| 130 | 0.07 | 0.02 |
| 140 | 0.07 | 0.02 |
| 150 | 0.07 | 0.02 |

At least 100 (constant density grid with charged particle implementation) or 146 (constant step size grid with 20deg step size) measurement grid points shall be used for EIRP spherical coverage procedure. Compared with 8x2 antenna array, the factor of improvement based on new measurement grid with 4x2 antenna is about 2, as summarized in the table 8.2.1.2-3.

Table 8.2.1.2-3: Min Number of Grid Points for Spherical Coverage

| Antenna Assumption Grid Type | 8x2 | 4x2 | Factor of Improvement |
| --- | --- | --- | --- |
| Constant-Step Size | 266 (15.0 deg) | 146 (20.0 deg) | 1.8 |
| Constant-Density | 200 | 100 | 2 |

#### 8.2.1.3	TRP Measurement Grid

The simulation assumptions including the antenna patterns for the TRP measurement grids are the same as Clause G.2 [3] except a 4x2 antenna array assumption instead of 8x2 for both single-element antenna patterns.

The results tabulated in this section outline the results of a statistical analyses with the positioning concept taken into account, i.e., the analyses were performed with and without the assumption that the beam peak direction is oriented away from the hemisphere towards the pole at = 180o. Additionally, the standard deviations are presented when ranges of pattern values are disregarded (zeroed out). For the constant-step size measurement grids, three cases were investigated, i.e., no pattern values are disregarded, values only at one latitude at =180o, and the values at the bottom two latitudes are disregarded. The results with the re-positioning concept applied are summarized in Table 8.2.1.3-1 for the sin(theta) and the Clenshaw-Curtis quadratures while the results without the re-positioning concept applied are summarized in Table 8.2.1.3-2.

For the constant density measurement grids, a similar investigation was performed using the Charged Particle implementation. Two cases investigated were: no pattern values are disregarded and values betweenX ≤ ≤ 180o are disregarded. The results with the re-positioning concept applied are summarized in Table 8.2.1.3-2 for the Charged Particle implementation while the results without the re-positioning concept applied are summarized in Table 8.2.1.3-4.

The previously agreed limit for the PC3 TRP grids is 0.25dB. Those measurement grids meeting that limit have been highlighted in green while the grids exceeding that limit are highlighted in red. It should be noted that some mean errors are relatively high for grids that meet the 0.25dB std. deviation limit and therefore should not be considered candidate measurement grids.

Table 8.2.1.3-1: Statistics of quadrature approaches for constant step size measurement grids for the 4x2 antenna array with the re-positioning concept applied.

| Number of |  | Step Size = | Number of unique grid points | Number of Latitudes disregarded | Mean Error [dB] | Std. Dev [dB] | Quadrature | Re-Positioning Concept Applied |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Latitudes | Longitudes |  |  |  |  |  |  |  |
| 13 | 24 | 15 | 266 | 1 | -0.02 | 0.05 | Sin(theta) | yes |
| 13 | 24 | 15 | 266 | 1 | -0.01 | 0.01 | Clenshaw-Curtis | yes |
| 13 | 24 | 15 | 266 | 2 | -0.10 | 0.16 | Sin(theta) | yes |
| 13 | 24 | 15 | 266 | 2 | -0.08 | 0.12 | Clenshaw-Curtis | yes |
| 13 | 24 | 15 | 266 | 3 | -0.18 | 0.17 | Sin(theta) | yes |
| 13 | 24 | 15 | 266 | 3 | -0.16 | 0.14 | Clenshaw-Curtis | yes |
| 10 | 18 | 20 | 146 | 1 | -0.05 | 0.10 | Sin(theta) | yes |
| 10 | 18 | 20 | 146 | 1 | -0.01 | 0.03 | Clenshaw-Curtis | yes |
| 10 | 18 | 20 | 146 | 2 | -0.19 | 0.27 | Sin(theta) | yes |
| 10 | 18 | 20 | 146 | 2 | -0.15 | 0.18 | Clenshaw-Curtis | yes |
| 10 | 18 | 20 | 146 | 3 | -0.31 | 0.21 | Sin(theta) | yes |
| 10 | 18 | 20 | 146 | 3 | -0.28 | 0.17 | Clenshaw-Curtis | yes |
| 8 | 14 | 25.71 | 86 | 1 | -0.08 | 0.19 | Sin(theta) | yes |
| 8 | 14 | 25.71 | 86 | 1 | -0.02 | 0.05 | Clenshaw-Curtis | yes |
| 8 | 14 | 25.71 | 86 | 2 | -0.32 | 0.40 | Sin(theta) | yes |
| 8 | 14 | 25.71 | 86 | 2 | -0.25 | 0.26 | Clenshaw-Curtis | yes |
| 8 | 14 | 25.71 | 86 | 3 | -0.52 | 0.24 | Sin(theta) | yes |
| 8 | 14 | 25.71 | 86 | 3 | -0.46 | 0.17 | Clenshaw-Curtis | yes |
| 7 | 12 | 30 | 62 | 1 | -0.11 | 0.33 | Sin(theta) | yes |
| 7 | 12 | 30 | 62 | 1 | -0.03 | 0.13 | Clenshaw-Curtis | yes |
| 7 | 12 | 30 | 62 | 2 | -0.44 | 0.53 | Sin(theta) | yes |
| 7 | 12 | 30 | 62 | 2 | -0.34 | 0.32 | Clenshaw-Curtis | yes |
| 7 | 12 | 30 | 62 | 3 | -0.73 | 0.36 | Sin(theta) | yes |
| 7 | 12 | 30 | 62 | 3 | -0.66 | 0.26 | Clenshaw-Curtis | yes |

Table 8.2.1.3-2: Statistics of quadrature approaches for constant step size measurement grids for the 4x2 antenna array without the re-positioning concept applied.

| Number of |  | Step Size = | Number of unique grid points | Number of Latitudes disregarded | Mean Error [dB] | Std. Dev [dB] | Quadrature | Re-Positioning Concept Applied |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Latitudes | Longitudes |  |  |  |  |  |  |  |
| 13 | 24 | 15 | 266 | 1 | -0.03 | 0.06 | Sin(theta) | no |
| 13 | 24 | 15 | 266 | 1 | -0.02 | 0.04 | Clenshaw-Curtis | no |
| 13 | 24 | 15 | 266 | 2 | -0.19 | 0.37 | Sin(theta) | no |
| 13 | 24 | 15 | 266 | 2 | -0.18 | 0.35 | Clenshaw-Curtis | no |
| 10 | 18 | 20 | 146 | 1 | -0.05 | 0.10 | Sin(theta) | no |
| 10 | 18 | 20 | 146 | 1 | -0.03 | 0.07 | Clenshaw-Curtis | no |
| 10 | 18 | 20 | 146 | 2 | -0.35 | 0.63 | Sin(theta) | no |
| 10 | 18 | 20 | 146 | 2 | -0.33 | 0.59 | Clenshaw-Curtis | no |
| 8 | 14 | 25.71 | 86 | 1 | -0.08 | 0.20 | Sin(theta) | no |
| 8 | 14 | 25.71 | 86 | 1 | -0.05 | 0.12 | Clenshaw-Curtis | no |
| 8 | 14 | 25.71 | 86 | 2 | -0.62 | 0.96 | Sin(theta) | no |
| 8 | 14 | 25.71 | 86 | 2 | -0.56 | 0.90 | Clenshaw-Curtis | no |
| 7 | 12 | 30 | 62 | 1 | -0.11 | 0.33 | Sin(theta) | no |
| 7 | 12 | 30 | 62 | 1 | -0.07 | 0.22 | Clenshaw-Curtis | no |
| 7 | 12 | 30 | 62 | 2 | -0.87 | 1.24 | Sin(theta) | no |
| 7 | 12 | 30 | 62 | 2 | -0.79 | 1.15 | Clenshaw-Curtis | no |

Table 8.2.1.3-3: Statistics for constant density measurement grid types for the 4x2 reference antenna array with the re-positioning concept applied (charged particle implementation only)

| Number of Grid Points | Range of Angles disregarded | Mean Error [dB] | Std. Dev [dB] | Re-Positioning Concept Applied |
| --- | --- | --- | --- | --- |
| 90 | none | 0.05 | 0.02 | yes |
| 80 | none | 0.05 | 0.03 | yes |
| 70 | none | 0.05 | 0.03 | yes |
| 60 | none | 0.05 | 0.05 | yes |
| 50 | none | 0.05 | 0.07 | yes |
| 40 | none | 0.04 | 0.17 | yes |
| 90 | 165o-180o | 0.00 | 0.08 | yes |
| 80 | 165o-180o | -0.01 | 0.09 | yes |
| 70 | 165o-180o | 0.02 | 0.07 | yes |
| 60 | 165o-180o | 0.01 | 0.09 | yes |
| 50 | 165o-180o | 0.00 | 0.11 | yes |
| 40 | 165o-180o | 0.04 | 0.17 | yes |
| 90 | 150o-180o | -0.10 | 0.18 | yes |
| 80 | 150o-180o | -0.09 | 0.18 | yes |
| 70 | 150o-180o | -0.11 | 0.20 | yes |
| 60 | 150o-180o | -0.10 | 0.20 | yes |
| 50 | 150o-180o | -0.14 | 0.21 | yes |
| 40 | 150o-180o | -0.13 | 0.28 | yes |

Table 8.2.1.3-4: Statistics for constant density measurement grid types for the 4x2 reference antenna array without the re-positioning concept applied (charged particle implementation only)

| Number of Grid Points | Range of Angles disregarded | Mean Error [dB] | Std. Dev [dB] | Re-Positioning Concept Applied |
| --- | --- | --- | --- | --- |
| 90 | none | 0.05 | 0.02 | no |
| 80 | none | 0.05 | 0.03 | no |
| 70 | none | 0.05 | 0.03 | no |
| 60 | none | 0.05 | 0.05 | no |
| 50 | none | 0.05 | 0.07 | no |
| 40 | none | 0.05 | 0.17 | no |
| 90 | 165o-180o | 0.00 | 0.13 | no |
| 80 | 165o-180o | -0.01 | 0.14 | no |
| 70 | 165o-180o | -0.02 | 0.17 | no |
| 60 | 165o-180o | -0.03 | 0.21 | no |
| 50 | 165o-180o | -0.04 | 0.26 | no |
| 40 | 165o-180o | -0.08 | 0.39 | no |
| 90 | 150o-180o | -0.28 | 0.58 | no |
| 80 | 150o-180o | -0.26 | 0.56 | no |
| 70 | 150o-180o | -0.32 | 0.64 | no |
| 60 | 150o-180o | -0.29 | 0.61 | no |
| 50 | 150o-180o | -0.36 | 0.70 | no |
| 40 | 150o-180o | -0.34 | 0.73 | no |

According to the above analysis, the following conclusions can be made:

-	If the re-positioning concept is not applied to TRP test cases:

-	40 measurement grid points for constant density grid – Charged Particle implementation, with standard deviation of 0.17dB.

-	8 latitudes and 14 longitudes (84 grid points) for constant step size grid – sin (theta) weights integration approach, with standard deviation of 0.20dB with the allowance to skip and interpolate measurements at the pole at =180o.

-	7 latitudes and 12 longitudes (62 grid points) for constant step size grid – Clenshaw Curtis weights integration approach, with standard deviation of 0.22 dB with the allowance to skip and interpolate measurements at the pole at =180o

-	If the re-positioning concept is applied to TRP test cases:

-	50 measurement grid points for constant density grid – Charged Particle implementation, with standard deviation of 0.21 dB with the allowance to skip and interpolate measurements beyond 150o in 

-	8 latitudes and 14 longitudes (86 grid points) for constant step size grid – sin (theta) weights integration approach, with standard deviation of 0.19dB with the allowance to skip and interpolate measurements the at pole at =180o

-	7 latitudes and 12 longitudes (62 grid points) for constant step size grid – Clenshaw Curtis weights integration approach, with standard deviation of 0.13 dB with the allowance to skip and interpolate measurements the at pole at =180o

### 8.2.2	Applicability of the 4x2 measurement grids

Since RAN5 has decided on maximum test system uncertainties and test tolerances already, it is not suggested to change the assumptions at this point as this will have significant impact in RAN5 and industry since changes in MU/MTSU could have impact on certifications and test platform validations. Keep the system-related assumptions unchanged in RAN5, i.e., based on the previously agreed worst case 8x2 assumptions.

It is therefore the 4x2-antenna-based measurement grids are agreed as an additional option for FR2 test cases, but not replace previous 8x2 based measurement grids. The selection of measurement grid based on 4x2 or 8x2 is based on optional vendor declaration.

The above new measurement grids based on 4x2 antenna array are applicable to both NTC and ETC test cases.

## 8.3	RSRP(B) based RX beam peak search

RSRP(B)-based RX beam peak search approach is applicable to find the beam peak, the beam peak searching time can be reduced significantly.

### 8.3.1	Test procedure

The RX beam peak direction is found with a 3D RSRP(B) scan (separately for each orthogonal downlink polarization). The RX beam peak direction is where the maximum total component of RSRP is found. The RX beam peak direction search grid points for this single grid approach are defined in Clause 8.2.

The measurement procedure includes the following steps:

1)	Select any of the three Alignment Options (1, 2, or 3) from Tables N.2-1 through N.2-3 [6] to mount the DUT inside the QZ.

2)	Position the DUT in DUT Orientation 1 or 2 from Tables N.2-1 through N.2-3 [6].

3)	Connect the SS (System Simulator) with the DUT through the measurement antenna with PolLink= polarization to form the RX beam towards the measurement antenna.

4)	Set a proper high DL power supported by the test system, this value will be defined in RAN5 conformance test spec. Determine RSRP or RSRPBs (one per receiver branch) at PolMeas=PolLink=condition reported by UE.

5)	Connect the SS (System Simulator) with the DUT through the measurement antenna with PolLink= polarization to form the RX beam towards the measurement antenna.

6)	Set the same DL power as the one in step 4. Determine RSRP or RSRPBs (one per receiver branch) at PolMeas=PolLink=condition reported by UE.

7)	Advance to the next grid point and repeat steps 3 through 6 until measurements within the full 3D scan have been completed.

8)	Data processing the linear sum of four reported RSRPBs. How to calculate the reported RSRPs is FFS.

Note: RSRPB-based test procedure is adopted for RX beam peak.  Feasibility of RSRP-based approach is FFS.

### 8.3.2	RSRP(B) accuracy

To guarantee RSRP(B) accuracy, SNR side condition configuration can refer to the minimum SSB_RP specified for beam correspondence in TS 38.101-2.

## 8.4	Single link polarization measurement

As an enhancement to the FR2 2Tx test cases, it has been proposed to adopt a Single link polarization measurement to reduce the test time. Single Pollink can be randomly selected from either theta Pollink or phi Pollink.

For EIRP test, whether single Pollink is adopted or test under 2 link directions, depends on UE declaration.

### 8.4.1	Test procedure

For single link polarization measurement, the link antenna can be randomly selected, in this clause the detailed Single Pollink measurement procedure for TX Beam Peak direction search and EIRP Spherical Coverage based on PolLink= is presented as an example:

1)	Select any of the three Alignment Options (1, 2, or 3) from Tables N.2-1 through N.2-3 [6] to mount the DUT inside the QZ.

2)	Position the DUT in DUT Orientation 1 from Tables N.2-1 through N.2-3 [6].

3)	Connect the SS (System Simulator) with the DUT through the measurement antenna with PolLink= to form the TX beam towards the measurement antenna.

4)	DUT refines its TX beam toward that direction depending on DUT’s beam correspondence capability which shall match OEM declaration:

a)	if  DUT’s beam correspondence capability is [bit-1], then DUT autonomously chooses the corresponding TX beam for PUSCH transmission using downlink reference signals to transmit in the direction of the incoming DL signal, which is based on beam correspondence without relying on UL beam sweeping

b)	if DUT’s beam correspondence capability is [bit-0], then DUT chooses the TX beam for PUSCH transmission which is based on beam correspondence that relies on both DL measurements on downlink reference signals and network-assisted uplink beam sweeping.

5)	Lock the beam and send continuously power control "up" commands in every uplink scheduling information to the UE

6)	Measure the mean power Pmeas(PolMeas=PolLink=) of the modulated signal arriving at the power measurement equipment (such as a spectrum analyser, power meter, or gNB emulator).

7)	Calculate EIRP (PolMeas=PolLink=) by adding the composite loss of the entire transmission path for utilized signal path, LEIRP,θ, and frequency to the measured power Pmeas(PolMeas=PolLink=)

8)	Measure the mean power Pmeas (PolMeas=PolLink=) of the modulated signal arriving at the power measurement equipment.

9)	Calculate EIRP (PolMeas=PolLink=) by adding the composite losses of the entire transmission path for utilized signal path, LEIRP,ϕ, and frequency to the measured power Pmeas (PolMeas=PolLink=)

10)	Calculate total EIRP(PolLink=) = EIRP(PolMeas=PolLink=) + EIRP(PolMeas=PolLink=)

11)	Advance to the next grid point and repeat steps 3 through 13 until measurements within zenith range 0o≤≤90o have been completed

12)	After the measurements within zenith range 0o≤≤90o have been completed and

a)	if the re-positioning concept is applied to the TX test cases, position the device in DUT Orientation 2 (either Options 1 or 2) from Tables N.2-1 through N.2-3 [6] for the Alignment Option selected in Step 1. For the TX beam peak search in the second hemisphere, perform steps 3 through 14 for the range of zenith angles 90o<≤0o.

b)	if the re-positioning concept is not applied to the TX test cases, continue steps 3 through 13 for the range of zenith angles 90o<≤180o

The TX beam peak direction is where the maximum total component of EIRP(PolLink=) is found.

The EIRPtarget-CDF is then obtained from the Cumulative Distribution Function (CDF) computed using maximum EIRP(PolLink=) for all grid points.

### 8.4.2	Applicability of Single link polarization measurement

Whether single Pollink is adopted or test under 2 link directions, depends on UE declaration. Link antenna for Single Pollink measurement can be randomly selected, i.e., using either theta Pollink or phi Pollink.

## 8.5	Other methods

### 8.5.1	Fast Spherical Coverage Method

#### 8.5.1.1	General

The Fast Spherical Coverage Method is a test method providing an optimized test time for Tx and Rx spherical coverage measurements.

Instead of measuring all grid points as defined in Annex M of TS 38.521-2 [6] as required by the current test procedure as defined in Annex K.1.5 and Annex K.1.6 of TS 38.521-2 [6], the Fast Spherical Coverage Method requires only a reduced number of grid points to be measured.

For test systems where the device repositioning approach outlined in Annex N of TS 38.521-2 [6] is applied, the grid points of up to a zenith of [90]° are allowed to be measured in the first hemisphere before the device needs to be placed in the second orientation.

This method is applicable to Constant Density grid type. Fast spherical coverage measurement method is also applicable for constant step size grid type.

#### 8.5.1.2	Tx Fast Spherical Coverage Method

The measurement procedure for an EIRP Fast Spherical Coverage Method includes the following steps:

1)	Select any of the three Alignment Options (1, 2, or 3) from Tables N.2-1 through N.2-3 [6] to mount the DUT inside the QZ.

2) 	Position the DUT in DUT Orientation 1 or 2 from Tables N.2-1 through N.2-3 [6].

3) 	Connect the SS (System Simulator) with the DUT through the measurement antenna with PolLink=θ polarization to form the TX beam towards the measurement antenna.

4)	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE.

5) 	For beam correspondence, DUT refines its TX beam toward that direction depending on DUT’s beam correspondence capability which shall match OEM declaration.

6) 	Lock the beam using the UE beamlock function.

7)	Measure the mean power Pmeas (PolMeas= θ, PolLink= θ) of the modulated signal arriving at the power measurement equipment (such as a spectrum analyser, power meter, or gNB emulator).

8)	Calculate EIRP (PolMeas= θ, PolLink= θ)  by adding the composite loss of the entire transmission path for utilized signal path, LEIRP,θ, and frequency to the measured power Pmeas(PolMeas= θ, PolLink= θ).

9)	Measure the mean power Pmeas (PolMeas= ϕ, PolLink= θ) of the modulated signal arriving at the power measurement equipment.

10)	Calculate EIRP (PolMeas= ϕ, PolLink= θ) by adding the composite losses of the entire transmission path for utilized signal path, LEIRP,ϕ, and frequency to the measured power Pmeas (PolMeas= ϕ, PolLink= θ).

11)	Calculate total EIRP(PolLink= θ)  = (PolMeas= θ, PolLink= θ)  + EIRP(PolMeas= ϕ, PolLink= θ).

12)	Unlock the beam using the UE beamlock function.

13)	Connect the SS (System Simulator) with the DUT through the measurement antenna with PolLink= ϕ polarization to form the TX beam towards the measurement antenna and repeat steps 4 through 12.

14) Calculate the EIRP result for the grid point as EIRPspherical = Max(EIRP(PolLink= θ), EIRP(PolLink= ϕ)). If the EIRPspherical value is above the Min EIRP spherical coverage limit increase Ngrid, meas, PASS by 1.

15) Calculate the percentage of total grid points measured so far above the EIRP spherical coverage requirement limit Ngrid, meas, PASS compared to the total number of grid points on the measurement grid Ngrid,total.

16) If the percentage calculated in step 15 is equal to or higher than (100 - nth percentile for EIRP spherical coverage)%, pass the device, otherwise continue to step 17. If all grid points have been measured, calculate the CDF for all grid points and pass the UE if the derived %-tile EIRP in measurement distribution exceeds the requirement. Otherwise fail the UE.

17) Advance to the next grid point and repeat steps 3 through 16 until measurements within zenith range 0º≤ θ ≤[90]º have been completed

18)	After the measurements within zenith range 0º≤ θ ≤[90]º have been completed and

a)	if the re-positioning concept is applied to the TX test cases, position the device in the corresponding second DUT Orientation from Tables N.2-1 through N.2-3 [6] for the Alignment Option selected in Step 1 and DUT Orientation selected in Step 2. For the TX spherical coverage measurement in the second hemisphere, perform steps 3 through 16 for the range of zenith angles [90]º< θ ≤0º.

b)	if the re-positioning concept is not applied to the TX test cases, continue steps 3 through 16 for the range of zenith angles [90]º< θ ≤180º.

#### 8.5.1.3	Rx Fast Spherical Coverage Method

The measurement procedure for an EIS Fast Spherical Coverage Method includes the following steps:

1)	Select any of the three Alignment Options (1, 2, or 3) from Tables N.2-1 through N.2-3 [6] to mount the DUT inside the QZ.

2) 	Position the DUT in DUT Orientation 1 or 2 from Tables N.2-1 through N.2-3 [6].

3) 	Connect the SS (System Simulator) with the DUT through the measurement antenna with PolLink= θ polarization to form the RX beam towards the measurement antenna.

4)	Determine EIS(PolMeas= θ PolLink= θ) for θ-polarization, i.e., the power level for the θ-polarization at which the throughput exceeds the requirements for the specified reference measurement channel. The downlink power step size shall be no more than 0.2 dB when the RF power level is near the sensitivity level.

5)	Connect the SS (System Simulator) with the DUT through the measurement antenna with PolLink= ϕ polarization to form the RX beam towards the RX beam peak direction.

6)	Determine EIS(PolMeas= ϕ PolLink= ϕ) for ϕ-polarization, i.e., the power level for the ϕ-polarization at which the throughput exceeds the requirements for the specified reference measurement channel. The downlink power step size shall be no more than 0.2 dB when the RF power level is near the sensitivity level.

7)	Calculate the resulting averaged EIS as: EIS = 2*[1/EIS(PolMeas= θ PolLink= θ) +1/EIS(PolMeas= ϕ PolLink= ϕ)]-1. If the EIS value is below the EIS spherical coverage limit increase Ngrid, meas, PASS by 1.

8) Calculate the percentage of total grid points measured so far below the EIS spherical coverage requirement limit Ngrid, meas, PASS compared to the total number of grid points on the measurement grid Ngrid,total.

9) If the percentage calculated in step 8 is equal to or higher than (100 - nth percentile for EIS spherical coverage)%, pass the device, otherwise continue to step 10. If all grid points have been measured, calculate the CCDF for all grid points and pass the UE if the derived %-tile EIS in measurement distribution is lower than the requirement. Otherwise fail the UE.

10) Advance to the next grid point and repeat steps 3 through 16 until measurements within zenith range 0º≤ θ ≤[90]º  have been completed

11)	After the measurements within zenith range 0º≤ θ ≤[90]º have been completed and

a) if the re-positioning concept is applied to the TX test cases, position the device in the corresponding second DUT Orientation from Tables N.2-1 through N.2-3 [6] for the Alignment Option selected in Step 1 and DUT Orientation selected in Step 2. For the RX spherical coverage measurement in the second hemisphere, perform steps 3 through 9 for the range of zenith angles [90]º< θ ≤0º.

b) if the re-positioning concept is not applied to the RX test cases, continue steps 3 through 9 for the range of zenith angles [90]º< θ ≤180º.

### 8.5.2	Non-Uniform TRP Measurement Grids

Given the large number of PC1 TRP grid points, as specified in Clause M.4.1.1 of [6], additional test time reduction can be achieved based on non-uniform grids. The idea here is to apply a fine grid around the FF beam peak direction to capture the main portion of the very directive beam while a coarse grid around the remaining portion of the sphere is applied. This is further illustrated in Figure 8.5.2-1 for a constant step-size grid with the following non-uniform TRP grid assumptions:

-	The known FF beam is shown with the large grey dot. On top, the FF beam peak is assumed at (0o,0o) while the FF beam peak on the bottom is assumed at (45o,45o).

-	The red grid points are within a ±20o cone centred around the NF beam peak with ==7.5o.

-	The cyan grid points are outside a ±20o cone centred around the NF beam peak with ==15o.

/

Figure 8.5.2-1: Visualization of non-uniform TRP grids for NF beam at (0o,0o) on top and at (45o,45o) on bottom. Grid points in cyan (red) are outside (inside) the conical NF beam peak region.

Simulations with 2000 random permutations of the beam peak direction (rotation in , , and twist  as outlined in Clause G.1 of [3]) were performed for a FF range length of 20m; only the Clenshaw-Curtis quadrature was considered here. Table 8.5.2-1 shows the simulation results for the non-uniform measurement grids considered suitable for PC1 devices; the grid with step size of 7.5o within ±20o of the FF beam peak and step size of 15o outside ±20o of the FF beam peak is meeting the same MUs are the current measurement grid. The average number of unique grid points based on all simulations investigated is ~300 which shows a significant test time reduction compared to the current standard TRP grid with 1106 grid points while maintaining the same MU as the grid with uniform step sizes in  and .

Table 8.5.2-1: CFFDNF TRP simulation results (using Clenshaw-Curtis quadrature) for PC1 devices (12x12 antenna configuration) using non-uniform measurement grids.

| Antenna Configuration | Cone width (±) [o] | Constant Step-Size Grid Step Size outside cone = [o] | Constant Step-Size Grid Step Size within cone = [o] | Average number of unique grid points | ¦Mean TRP Error¦ [dB] | TRP Std. Dev. [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 12x12 | 15 | 10 | 5 | 648 | 0.1 | 0.1 |
|  | 20 | 15 | 5 | 338 | 0.1 | 0.1 |
|  |  | 15 | 7.5 | 293 | 0.1 | 0.1 |

The TRP calculation is left to the system vendor as there are different approaches to determine TRP, e.g., interpolation of all results to the fine grid vs partial TRPs calculated within the cone and outside the cone.

The comparison of the TRP measurement grid parameters and the min. number of grid points are tabulated in Table 8.5.2-2 with angular grid spacings placed uniformly and non-uniformly in  and . Clearly, the non-uniform TRP measurement grid approach is beneficial in terms of test time reduction.

Table 8.5.2-2: Comparison of the TRP measurement grid parameters for PC1 including potential test time improvement.

| Antenna Config. | Non-uniform angular spacing |  |  |  | Uniform angular spacing |  | Potential Test Time Improvement with non-uniform angular spacing in  and  (factor) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Cone width (±) [o] | Constant Step-Size Grid Step Size outside cone = [o] | Constant Step-Size Grid Step Size within cone = [o] | Average Number of unique grid points | Constant Step-Size Grid Step Size = [o] | Number of unique grid points |  |
| 12x12 | 20 | 15 | 7.5 | 293 | 7.5 | 1106 | 3.8 |

This non-uniform TRP grid approach is also applicable to the constant-density measurement grid type.

# 9	Propagation conditions

## 9.1	Extension of frequency applicability for FR2-2

Unless otherwise stated, the propagation condition methodology defined in TR 38.810 Clause 8 [3] is applicable for FR2-2:

-	Static propagation conditions

-	Model is described in TR 38.810 clause 8.3.

-	Applicable to the UE RRM testing methodology and UE Demodulation and CSI testing methodology.

-	Multi-path fading propagation conditions

-	Fading propagation conditions between the DUT and the emulated gNB sources are modelled as Single probe channel models as described in TR 38.810 clause 8.2.

-	Applicable to the UE RRM testing methodology and UE Demodulation and CSI testing methodology.

-	For FR2-2 measurement system is expected to support modelling of multi-path fading for single carrier scenarios with channel bandwidth of at least [400] MHz.

-	For FR2-2 single carrier scenarios with channel bandwidth up to [400] MHz, the Fsample is defined as [400] MHz and ∆T ≤ [2.5] ns.

-	An example for mapping the delays for a scaled power delay profile to the equidistant delay grid is given in clause 9.1.1

9.1.1	Example for determining the resulting delay profile

A power delay profile after delay spread scaling is given as follows in Table 9.1.1-1.

Table 9.1.1-1: Original power delay profile

| Tap k | Power σk2σk2 (linear) | Delay τk [ns] |
| --- | --- | --- |
| 1 | σ12 | 0 |
| 2 | σ22 | 1.91 |
| 3 | σ32 | 2.01 |
| 4 | σ42 | 2.93 |
| 5 | σ52 | 2.31 |
| 6 | σ62 | 2.69 |
| 7 | σ72 | 3.35 |
| 8 | σ82 | 2.88 |
| 9 | σ92 | 3.81 |
| 10 | σ102 | 7.69 |

With the assumption of ∆T = 1/(400 MHz) =2.5 ns, the taps from Table 9.1.1-1 can be mapped onto an equidistant delay grid as shown in Table 9.1.1-2.

Table 9.1.1-2: Power delay profile after mapping to delay grid

| Tap k | Power σk2σk2 (linear) | Delay τkτk [ns] |
| --- | --- | --- |
| 1 | σ12 | 0 |
| 2 | σ22 | 2.50 |
| 3 | σ32 | 2.50 |
| 4 | σ42 | 2.50 |
| 5 | σ52 | 2.50 |
| 6 | σ62 | 2.50 |
| 7 | σ72 | 3.75 |
| 8 | σ82 | 2.50 |
| 9 | σ92 | 3.75 |
| 10 | σ102 | 7.5 |

Since multiple taps share the same delay, those taps need to be combined into a single tap as shown in Table 9.1.1-3.

Table 9.1.1-3: Resulting delay profile

| Tap k | Power σk2σk2 (linear) | Delay τkτk [ns] |
| --- | --- | --- |
| 1 | σ12 | 0 |
| 2 | σ22+σ32+σ42+σ52+σ62+σ82 | 2.50 |
| 3 | σ72+σ92 | 3.75 |
| 4 | σ102 | 7.50 |

Annex A:
Environment conditions

# A.1	Operating voltage

# A.2	Temperature

Table A.2-1: Temperature conditions

| + 25 ⁰C ± 10 ⁰C | For normal (room temperature) conditions with relative humidity of 25 % to 75 % |
| --- | --- |
| -10C to +55C | For extreme conditions |

For ETC test system defined in Clause 5.4, the extreme temperature conditions in Table A.2-1 applies.

Annex B:
Measurement uncertainty

# B.1	Measurement uncertainty budget for UE RF testing methodology

Editor’s note: collect the MU elements which are impacted by the enhancements in Clauses 5 and 6 in this clause; if impact on the MU budget of the RRM and/or demodulation setups is identified, the corresponding clauses can be added. Organize the Annex to mirror the TR38.810 structure

## B.1.1	High DL power and low UL power

Editor’s note: the conclusion of MU impacts of the enhanced test methods (i.e. direct Near Field (DNF), Combined Far-Field/Direct Near Field (CFFDNF), and Combined Far-Field/Near Field (CFFNF)) should be captured.

### B.1.1.1	Uncertainty Contributions

This section covers the additional MU elements for different enhanced test systems in the following tables.

Table B.1.1.3-1: Uncertainty contributions for CFFNF system for PC3 UEs utilizing the black-box approach

| UID | Description of uncertainty contribution | Uncertainty Value | Details in Clause |
| --- | --- | --- | --- |
| Measurement stage |  |  |  |
| 1 | DUT antenna location estimation | 0 (Note 1) | N/A |
| 2 | Probe antenna pattern | 0 | B.1.1.2.2 |
| 3 | EIRP measurement error in NF | 0.27 (Note 2) | 5.1.4.6 (Table 5.1.4.6-4) |
| 4 | Near-field interaction between probe antenna and DUT antenna | 0 | B.1.1.2.6 |
| 5 | Influence of power measurement uncertainty | FFS (Note 3) | 5.1.4.7 |
| Systematic Uncertainties |  |  |  |
| 6 | EIRP measurement error in NF | 0.10 | 5.1.4.6 (Table 5.1.4.6-4) |
| Note: The range length for CFFNF with black-box approach is reported for radius r3>r2>r1 and r3=r2+1 cm=r1+2 cm=22 cm. Note 1: Black-box does not require the declaration of the antenna offset Note 2: Probe antenna pattern must be compensated. Note 3: MU can be finalized in RAN5 based on the analysis framework in Clause 5.1.4.7. |  |  |  |

Table B.1.1.3-2: Uncertainty contributions for CFFNF system for PC1 UEs utilizing the black-box approach

| UID | Description of uncertainty contribution | Uncertainty Value | Details in Clause |
| --- | --- | --- | --- |
| Measurement stage |  |  |  |
| 1 | DUT antenna location estimation | 0 (Note 1) | N/A |
| 2 | Probe antenna pattern | 0 | B.1.1.2.2 |
| 3 | EIRP measurement error in NF | 0.52 (Note 2) | 5.1.4.6 (Table 5.1.4.6-4) |
| 4 | Near-field interaction between probe antenna and DUT antenna | 0 | B.1.1.2.6 |
| 5 | Influence of power measurement uncertainty | FFS (Note 3) | 5.1.4.7 |
| Systematic Uncertainties |  |  |  |
| 6 | EIRP measurement error in NF | 0.17 (Note 2) | 5.1.4.6 (Table 5.1.4.6-4) |
| Note: The range length for CFFNF with black-box approach is reported for radius r3>r2>r1 and r3=r2+1 cm=r1+2 cm=32 cm. Note 1: Black-box does not require the declaration of the antenna offset Note 2: Probe antenna pattern must be compensated. Note 3: MU can be finalized in RAN5 based on the analysis framework in Clause 5.1.4.7. |  |  |  |

Table B.1.1.3-3: Uncertainty contributions for CFFNF system for PC3 UEs utilizing the black&white-box approach

| UID | Description of uncertainty contribution | Uncertainty Value | Details in Clause |
| --- | --- | --- | --- |
| Measurement stage |  |  |  |
| 1 | DUT antenna location estimation | FFS (Note 1) | 5.1.4.10 |
| 2 | Probe antenna pattern | 0 | B.1.1.2.2 |
| 3 | EIRP measurement error in NF | 0.02 (Note 2) | 5.1.4.5 (Table 5.1.4.5-3) |
| 4 | Near-field interaction between probe antenna and DUT antenna | 0 | B.1.1.2.6 |
| 5 | Influence of power measurement uncertainty | 0.37 | 5.1.4.7 |
| Note: The range length for CFFNF with black&white-box approach is reported for radius r2>r1 and r2=r1+2 cm=22 cm. Note 1: Pending OEM feedback on typical offset estimation error. Analyses in Clause 5.1.4.9 Note 2: Probe antenna pattern and array offset must be compensated. |  |  |  |

Table B.1.1.3-4: Uncertainty contributions for CFFNF system for PC1 UEs utilizing the black&white-box approach

| UID | Description of uncertainty contribution | Uncertainty Value | Details in Clause |
| --- | --- | --- | --- |
| Measurement stage |  |  |  |
| 1 | DUT antenna location estimation | FFS (Note 1) | 5.1.4.10 |
| 2 | Probe antenna pattern | 0 | B.1.1.2.2 |
| 3 | EIRP measurement error in NF | 0.04 (Note 2) | 5.1.4.5 (Table 5.1.4.5-4) |
| 4 | Near-field interaction between probe antenna and DUT antenna | 0 | B.1.1.2.6 |
| 5 | Influence of power measurement uncertainty | FFS (Note 3) | 5.1.4.7 |
| Systematic Uncertainties |  |  |  |
| 6 | EIRP measurement error in NF | 0.12 (Note 2) | 5.1.4.5 (Table 5.1.4.5-4) |
| Note: The range length for CFFNF with black&white-box approach is reported for radius r2>r1 and r2=r1+2 cm=32 cm. Note 1: Pending OEM feedback on typical offset estimation error. Analyses in Clause 5.1.4.9 Note 2: Probe antenna pattern and array offset must be compensated. Note 3: MU can be finalized in RAN5 based on the analysis framework in Clause 5.1.4.7. |  |  |  |

Table B.1.1.3-5: Uncertainty contributions for CFFDNF system for PC3 UEs utilizing the black&white-box approach

| UID | Description of uncertainty contribution | Uncertainty Value | Details in Clause |
| --- | --- | --- | --- |
| Measurement stage |  |  |  |
| 1 | DUT antenna location estimation | FFS (Note 1) | 5.1.4.10 |
| 2 | Probe antenna pattern | 0 | B.1.1.2.2 |
| 3 | EIRP measurement error in NF | 0.02 (Note 2) | 5.1.4.4 (Table 5.1.4.4-8) |
| 4 | TRP measurement error in NF | 0.05 (Note 3) | 5.1.4.4 (Table 5.1.4.4-9 for uniform constant-step size grids; Table 5.1.4.4-12 for non-uniform constant-step size grids) |
| 5 | Near-field interaction between probe antenna and DUT antenna | 0 | B.1.1.2.6 |
| Note: The range length for CFFNF with black&white-box approach is reported for radius r=35 cm (EIRP) and r=20 cm (TRP). Note 1: Pending OEM feedback on typical offset estimation error. Analyses in Clause 5.1.4.9 Note 2: Probe antenna pattern and array offset must be compensated Note 3: Alternate minimum range lengths, need for offset compensation, and measurement grids defined in Clause 5.1.4.4 |  |  |  |

Table B.1.1.3-6: Uncertainty contributions for CFFDNF system for PC1 UEs utilizing the black&white-box approach

| UID | Description of uncertainty contribution | Uncertainty Value | Details in Clause |
| --- | --- | --- | --- |
| Measurement stage |  |  |  |
| 1 | DUT antenna location estimation | FFS (Note 1) | 5.1.4.10 |
| 2 | Probe antenna pattern | 0 | B.1.1.2.2 |
| 3 | EIRP measurement error in NF | 0.05 (Note 2) | 5.1.4.4 (Table 5.1.4.4-8) |
| 4 | TRP measurement error in NF | 0.04 (Note 3) | 5.1.4.4 (Table 5.1.4.4-10 for uniform constant-step size grids; Table 5.1.4.4-11 for non-uniform constant-step size grids) |
| 5 | Near-field interaction between probe antenna and DUT antenna | 0 | B.1.1.2.6 |
| Systematic Uncertainties |  |  |  |
| 6 | EIRP measurement error | 0.45 (Note 2) | 5.1.4.4 (Table 5.1.4.4-8) |
| Note: The range length for CFFNF with black&white-box approach is reported for radius r=45 cm (EIRP) and r=20 cm (TRP). Note 1: Pending OEM feedback on typical offset estimation error. Analyses in Clause 5.1.4.9 Note 2: Probe antenna pattern and array offset must be compensated Note 3: Alternative minimum range lengths, need for offset compensation, and measurement grids defined in Clause 5.1.4.4 |  |  |  |

Table B.1.1.3-7 Uncertainty contributions for CFFdeltaNF system for PC3 UEs utilizing the black-box approach

| UID | Description of uncertainty contribution | Uncertainty Value | Details in Clause |
| --- | --- | --- | --- |
| Measurement stage |  |  |  |
| 1 | DUT antenna location estimation | 0 | N/A |
| 2 | Probe antenna pattern | 0 | N/A |
| 3 | EIRP measurement error in NF | 0 (Note 1) | 5.1.4.8 (Table 5.1.4.8-1) |
| 4 | Near-field interaction between probe antenna and DUT antenna | 0 | N/A |
| Systematic Uncertainties |  |  |  |
| 5 | EIRP measurement error in NF | 0.14 (Note 1) | 5.1.4.8 (Table 5.1.4.8-1) |
| Note: The range length for CFFdeltaNF with black&white-box approach is reported for radius r=35 cm (EIRP). Alternate minimum range lengths in Clause 5.1.4.8 Note 1: MUs include effect due to pattern change based on antenna array configuration adjustment between high-UL and low-UL operation; the effect due to pattern change based on PA nonlinearities are FFS |  |  |  |

Table B.1.1.3-8: Uncertainty contributions for CFFdeltaNF system for PC1 UEs utilizing the black-box approach

| UID | Description of uncertainty contribution | Uncertainty Value | Details in Clause |
| --- | --- | --- | --- |
| Measurement stage |  |  |  |
| 1 | DUT antenna location estimation | 0 | N/A |
| 2 | Probe antenna pattern | 0 | N/A |
| 3 | EIRP measurement error in NF | 0 (Note 1) | 5.1.4.8 (Table 5.1.4.8-3) |
| 4 | EIRP measurement error in NF due to pattern changes caused by PA non-linearities | FFS | 5.1.4.8 |
| 5 | Near-field interaction between probe antenna and DUT antenna | 0 | N/A |
| Systematic Uncertainties |  |  |  |
| 6 | EIRP measurement error in NF | 0.5 (Note 1) | 5.1.4.8 (Table 5.1.4.8-3) |
| Note: The range length for CFFdeltaNF with black&white-box approach is reported for radius r=45 cm (EIRP). Alternate minimum range lengths in Clause 5.1.4.8 Note 1: MUs include effect due to pattern change based on antenna array configuration adjustment between high-UL and low-UL operation; the effect due to pattern change based on PA nonlinearities are FFS |  |  |  |

### B.1.1.2	Uncertainty Contributions descriptions

#### B.1.1.2.1	DUT antenna location estimation

This contribution originates from estimating the phase centre of the active antenna array and thus declaring the phase centre offset incorrectly in the vendor declaration for the black&white-box approach. The factors included in this uncertainty are the differences in path losses between the declare and the actual offsets and the difference in compensated probe pattern. This uncertainty is assumed to have a Gaussian distribution. This MU element is applicable to CFFDNF&CFFNF methodologies with the black&white-box approach.

Additional background: This MU element was analysed in Clause 5.1.4.9.

#### B.1.1.2.2	Probe antenna pattern

The probe pattern is assumed to be known so that the measurement in the near field can be corrected when the probe pattern needs to be compensated. There is no direct dependence between the UE pattern and the probe pattern in near field measurements. This uncertainty is assumed to have a Gaussian distribution. This MU element is applicable to the CFFDNF&CFFNF methodologies.

Additional background: This MU element is similar to the ‘A3-14 Probe pattern knowledge’ of the Near Field Test Range in TR 37.941 and should be considered 0 since the probe pattern can readily be measured.

#### B.1.1.2.3	EIRP measurement error in NF

The cause of this uncertainty contributor is due to the reduction of distance between the measurement antenna and the DUT from Far-Field to Near-Field. This uncertainty is assumed to have a Gaussian distribution. This MU element is applicable to the CFFDNF, CFFNF, and CFFdeltaNF methodologies. This MU element, if a non-insignificant mean error was identified, shall also be listed under System Uncertainties.

Additional background: This MU element was analysed in Clauses 5.1.4.4, 5.1.4.5, 5.1.4.6, 5.1.4.8.

For the CFFdeltaNF methodology, there is an additional impact due to changes in antenna pattern caused by PA non-linearities, etc. which is FFS.

Additional background: This effect was analysed in Clause 5.1.4.8.

#### B.1.1.2.4	TRP measurement error in NF

This contribution originates from calculating the Total Radiated Power based on EIRP measurements in the NF. This uncertainty is assumed to have a Gaussian distribution. This MU element is applicable to the CFFDNF methodology.

Additional background: This MU element was analysed in Clause 5.1.4.4.

#### B.1.1.2.5	Near-field interaction between probe antenna and DUT antenna

This uncertainty term is related to the coupling between the UE antenna and the measurement antenna in the near field at different distances. This uncertainty can be determined by multiple measurements of the DUT when at different distance from the probe. This uncertainty is assumed to have a Gaussian distribution. This MU element is applicable to the CFFDNF&CFFNF methodologies.

Additional background: This MU element is similar to the ‘A3-15 Multiple Reflections’ MU element of the Near Field Test Range in TR 37.941 and can be assumed to be 0 but needs to be evaluated for each system separately.

#### B.1.1.2.6	Influence of power measurement uncertainty

This contribution originates from relative power measurement uncertainties at different radii when applied to the asymptotic expansion transformation. This uncertainty is assumed to have a Gaussian distribution. This MU element is applicable to the CFFNF methodology.

Additional background: This MU element was analysed in Clause 5.1.4.7.

#### B.1.1.2.7	Influence of noise

This contributor describes an offset uncertainty factor caused by a noise floor especially in a case of low SNR. This contributor works as a bias to measured results only to a direction to increase values and thus this shall be included in the uncertainty budget table as a systematic uncertainty. The uncertainty value can be derived by the following equation for CFFDNF and CFFdeltaNF.

Influence of noise=10*log⁡(1+10-SNR10)

For the CFFNF methodology, the asymptotic expansion approach requires EIRP measurements of two (black&white-box) or three different radii (black-box approach) and thus different SNR conditions for each EIRP measurement. The influence of noise MU therefore requires an AWGN analyses with those varying SNR conditions applied to the signals.

The available SNR for the CFFDNF and CFFNF methodologies must take the reduction in free-space path losses due to measurements in the near field and the reduction of relaxations into account while making sure that the available SNR is above 6dB.

Additional background: This MU element was analysed in Clause 5.1.4.8

### B.1.1.3	Uncertainty assessment

FFS

## B.1.2	Polarization basis mismatch between the TE and DUT

For TPMI-based test method for EIRP measurement, the fixed TPMI index is used to configure the UE to ensure 1 layer 2 port transmission. The test setup and test procedure keep unchanged, thus no additional MU is identified for this enhanced test method.

## B.1.3	Inter-band (FR2+FR2) CA

For IFF-based off-focus test system defined in Clause 5.3, the Quality of quiet zone (QoQZ) will be impacted by both the main antenna and of the offset antenna. The preliminary QoQZ difference is analysed to present the measurement uncertainty induced by the off-focus test system.

The difference between the QoQZ of main antenna and of the offset antenna is summarized in Table B.1.3-1 and plotted in Figure B.1.3-1. Note that these values are specific to the feed antenna (amplitude taper) in this experiment and thus they may vary depending on an antenna pattern used by each test equipment vendor.

Table B.1.3-1: Estimation of QoQZ difference between main and offset antenna

|  | Estimation of QoQZ difference (EIRP) [dB] |  |  |
| --- | --- | --- | --- |
| δ [mm] | 23.45 GHz | 32.125 GHz | 40.8 GHz |
| 0 | 0.00 | 0.00 | 0.00 |
| 15 | 0.02 | 0.02 | 0.01 |
| 30 | 0.04 | 0.05 | 0.03 |
| 45 | 0.08 | 0.08 | 0.06 |
| 60 | 0.11 | 0.11 | 0.09 |
| 75 | 0.14 | 0.15 | 0.11 |

Figure B.1.3-1: Plot of QoQZ difference

## B.1.4	Test system for ETC

The ETC test method has been defined in Clause 5.4. The ETC test system will increase measurement uncertainty compared with NTC test system. For MOP-EIRP and REFSENS-EIS, the comparison of the MU under NTC and ETC is summarized in the table B.1.4-1.

Table B.1.4-1: Comparison of MOP-EIRP and REFSENS-EIS MTSUs.

| Test Case | MTSU [dB] |  |  |  |
| --- | --- | --- | --- | --- |
|  | NTC |  | ETC |  |
|  | FR2A (23.45GHz - 32.125GHz) | FR2B (32.125GHz - 40.8GHz) | FR2A (23.45GHz - 32.125GHz) | FR2B (32.125GHz - 40.8GHz) |
| MOP-EIRP | 4.89 | 5.09 | 5.17 | 5.37 |
| REFSENS-EIS | 5.19 | 5.19 | 5.45 | 5.45 |

Whether the recommended ±[4]ºC temperature tolerance limit of FR2 ETC system defined in clause 5.4.4 would introduce additional MU or not, is FFS.

## B.1.5	Measurement uncertainty budget for FR2-2 test methods

Editor’s note: based on the approved SID, the preliminary MU assessment for 52.6GHz should be added.

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-10 | R4#92bis | R4-1913071 |  |  |  | Initial skeleton | 0.0.1 |
| 2020-11 | R4#97e | R4-2017663 |  |  |  | Implemented the following text proposal: R4-2017598 TP to TR38.884 on High DL and Low UL power test cases | 0.1.0 |
| 2021-02 | R4#98e | R4-2103985 |  |  |  | Implemented the following text proposals: R4-2100530 TP to TR38.884 on structure updates related to band n262 R4-2103966 TP to TR 38.884 on Low UL Power/High DL Power Test Cases R4-2103967 TP to TR38.884 v0.1.0 on polarization basis mismatch R4-2103968 TP to TR 38.884 on Inter-band DL CA in FR2 | 0.2.0 |
| 2021-04 | R4#98bis | R4-2106165 |  |  |  | Implemented the following text proposals: R4-2104523 TP to TR38.884 v0.2.0 on MU Annex R4-2104897 Rapporteur input to TR38.884 R4-2104958 TP to TR 38.884 on Inter-band DL CA in FR2 R4-2106129 TP to TR38.884 v0.2.0 on ETC system R4-2104519 Discussion and TP to TR38.884 on FR2 test time reduction R4-2106164 Text proposal to TR38.884: Fast Spherical Coverage Method | 0.3.0 |
| 2021-05 | R4#99e | R4-2108098 |  |  |  | Implemented the following text proposals: R4-2108652 TP to TR38.884 v0.3.0 on UL signal demodulation R4-2108658 TP to TR38.884 v0.3.0 on Environment conditions R4-2108659 TP to TR38.884 v0.3.0 on measurement uncertainty R4-2108653 TP on high DL power and low UL power test cases R4-2108655 TP to TR38.884 v0.3.0 on testing time reduction R4-2108657 TP to TR38.884 on permitted test methods for demodulation and RRM in band n262 | 0.4.0 |
| 2021-06 | RP#92e | RP-211465 |  |  |  | Presented to RAN for information | 1.0.0 |
| 2021-08 | R4#100e | R4-2115820 |  |  |  | Implemented the following text proposals: R4-2112986 TP to TR38.884 on TPMI index for EIRP measurement R4-2115762 TP on high DL power and low UL power test cases R4-2115764 TP to TR38.884 on the EVM calculation method R4-2115766 TP to TR38.884 on enhanced test methods for FR2 testing time reduction R4-2115822 TR structure to accommodate OTA test methods for 52.6-71GHz | 1.1.0 |
| 2021-11 | R4#101e | R4-2120768 |  |  |  | Implemented the following text proposals: R4-2120697 TP on CFFdeltaNF Rohde & Schwarz, Keysight Technologies R4-2120696 Preliminary MU assessment of CFFDNF and CFFNF methodologies Keysight Technologies, Rohde & Schwarz R4-2120695 TP to TR 38.884 on general aspects vivo | 1.2.0 |
| 2022-01 | R4#101bis | R4-2203128 |  |  |  | Implemented the following text proposals: R4-2200452 TP to TR 38.884 on release independence applicability of test method enhancements R4-2203078 TP to TR 38. 884 on extension of NR test methods to FR2-2 | 1.3.0 |
| 2022-03 | R4#102 | R4-2207512 |  |  |  | Implemented the following text proposals: R4-2207202 TP for TR 38.884 on NR test methods extension to FR2-2 R4-2207203 TP to TR38.884 on minimum SNR for RRM test cases for band n263 R4-2207204 TP to TR38.884 on minimum SNR for demodulation test cases for band n263 | 1.4.0 |
| 2022-03 | RP#95e | RP-220452 |  |  |  | Presented to RAN for approval | 2.0.0 |
| 2022-03 | RP#95e | RP-220856 |  |  |  | Editorial corrections | 2.0.1 |