3GPP TR 38.886 V0.3.0 (2019-10)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

V2X Services based on NR;

User Equipment (UE) radio transmission and reception;

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<NR V2X (vehicle-to-everything), Sidelink>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions, symbols and abbreviations	7

3.1	Definitions	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Background	7

4.1	Justification	7

4.2	Objective	8

5	Evaluation of adjacent channel co-existence	10

5.1	Adjacent channel coexistence Scenarios	10

5.1.1	Coexistence scenarios	10

5.2	Coexistence simulation assumptions	12

5.2.1	Coexistence simulation parameters in ITS spectrum	12

5.2.1.1 Layout model	12

5.2.1.2 Simulation parameters	14

5.2.1.3 ACLR and ACS	14

5.2.2	Coexistence simulation parameters in licensed spectrum for FR1	16

5.2.2.1 Network layout model	16

5.2.2.2 Simulation parameters	18

5.2.2.3 ACLR and ACS	18

5.2.2.4 Power control	18

5.2.3	Coexistence simulation parameters in licensed spectrum for FR2	19

5.2.3.1 Network layout model	19

5.2.3.2 Simulation parameters	21

5.2.3.3 ACLR and ACS	21

5.2.3.4 Power control	21

5.2.4	PRR performance metrics for NR V2X coexistence simulation	22

5.3	Coexistence simulation results	22

5.3.1	Coexistence simulation results in ITS spectrum	22

5.3.2	Coexistence simulation results in licensed spectrum for FR1	22

5.3.3	Coexistence simulation results in licensed spectrum for FR2	22

6	Evaluation of In-device Coexistence	23

6.1 In-device coexistence scenarios	23

7	Operating bands and channel bandwidth	24

7.1	Operating bands	24

7.1.1	Operating bands in FR1	24

7.1.2	Operating bands in FR2	24

7.2	Channel bandwidth	24

7.2.1	Channel bandwidth in FR1	24

7.2.2	Channel bandwidth in FR2	24

8	Transmitter characteristics	25

8.1	NR V2X UE Tx requirements in FR1	25

8.2	NR V2X UE Tx requirements in FR2	25

9	Receiver characteristics	26

9.1	NR V2X UE Rx requirements in FR1	26

9.2	NR V2X UE Rx requirements in FR2	26

10	Tx/Rx characteristics of concurrent operation	27

10.1	NR V2X SL operation in 5.9GHz or other potential ITS spectrum and LTE/NR Uu operation in licensed band for FR1	27

10.2	NR V2X SL operation and LTE/NR Uu operation both in licensed band for FR1	27

10.3	NR V2X SL operation in 5.9GHz or other potential ITS spectrum and NR Uu operation in licensed bands for FR2	27

10.4	NR V2X SL operation and NR Uu operation both in licensed bands for FR2	27

11	Conclusion and recommendations	28

Annex A:	29

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

1	Scope

The present document is a technical report for NR V2X services with sidelink operation in Rel-16. The purpose is to specify radio solutions that are necessary for NR to support advanced V2X services (except the remote driving use case which was studied in TR 38.824) based on the study outcome captured in TR 38.885.

1. NR sidelink: Specify NR sidelink solutions necessary to support sidelink unicast, sidelink groupcast, and sidelink broadcast for V2X services, considering in-network coverage, out-of-network coverage, and partial network coverage.

The solutions should cover both the operating scenario where the carrier(s) is/are dedicated to V2X services and the operating scenario where the carrier(s) is/are licensed spectrum and also used for NR Uu/LTE Uu operation.

NR sidelink design starts with frequencies in FR1, and NR sidelink in FR2 is supported by applying the design for FR1 and PT-RS to the numerologies agreed for FR2. No FR2 specific optimization is supported in this WI except PT-RS. No beam management is supported in this work.

For the scenarios of NR sidelink carrier, this work will consider a single carrier for the NR sidelink transmission and reception.

In this work, 2Rx antennas as well as 4Rx antennas are supported. The full range of speeds defined in SA1 (TS 22.186) needs to be supported in FR1.

2. Specify support for NR Uu to provide control for LTE sidelink

3. Specify UE report to assist gNB scheduling

4. Specify support for QoS management

This Technical Report focus on NR sidelink UE Tx/Rx RF requirements and contains the adjacent channel coexistence evaluation results for NR sidelink operation at both licensed bands and the dedicated ITS spectrum for V2X service.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 30.007: "Guideline on WI/SI for new Operating Bands".

[3]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[4]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[5]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3.2	Symbols

For the purposes of the present document, the following symbols apply:

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

4	Background

4.1	Justification

To expand the 3GPP platform to the automotive industry, the initial standard on support of V2V services was completed in September 2016. Enhancements that focusing on additional V2X operation scenarios leveraging the cellular infrastructure, are completed in March 2017 as 3GPP V2X phase 1 for inclusion in Release 14 LTE.  In Rel-14 LTE V2X, a basic set of requirements for V2X service in TS 22.185 derived from TR 22.885 has been supported, which are considered sufficient for basic road safety service. Vehicles (i.e., UEs supporting V2X applications) can exchange their own status information through sidelink, such as position, speed and heading, with other nearby vehicles, infrastructure nodes and/or pedestrians.

3GPP V2X phase 2 in Rel-15 introduces a number of new features in sidelink, including: carrier aggregation, high order modulation, latency reduction, and feasibility study on both transmission diversity and short TTI in sidelink.  All these enhanced features in 3GPP V2X phase 2 are primary base on LTE and require co-existing with Rel-14 UE in same resource pool.

SA1 has completed enhancement of 3GPP support for V2X services (eV2X services) in Rel-15. The consolidated requirements for each use case group (see below) are captured in TR 22.886, and a set of the normative requirements are defined in TS 22.186 in Rel-15.

SA1 has identified 25 use cases for advanced V2X services and they are categorized into four use case groups: vehicles platooning, extended sensors, advanced driving and remote driving. The detailed description of each use case group is provided as below.

- Vehicles Platooning enables the vehicles to dynamically form a group travelling together. All the vehicles in the platoon receive periodic data from the leading vehicle, in order to carry on platoon operations. This information allows the distance between vehicles to become extremely small, i.e., the gap distance translated to time can be very low (sub second). Platooning applications may allow the vehicles following to be autonomously driven.

- Extended Sensors enables the exchange of raw or processed data gathered through local sensors or live video data among vehicles, RSUs, devices of pedestrians and V2X application servers. The vehicles can enhance the perception of their environment beyond what their own sensors can detect and have a more holistic view of the local situation.

- Advanced Driving enables semi-automated or fully-automated driving. Longer inter-vehicle distance is assumed. Each vehicle and/or RSU shares data obtained from its local sensors with vehicles in proximity, thus allowing vehicles to coordinate their trajectories or maneuvers. In addition, each vehicle shares its driving intention with vehicles in proximity. The benefits of this use case group are safer traveling, collision avoidance, and improved traffic efficiency.

- Remote Driving enables a remote driver or a V2X application to operate a remote vehicle for those passengers who cannot drive themselves or a remote vehicle located in dangerous environments. For a case where variation is limited and routes are predictable, such as public transportation, driving based on cloud computing can be used. In addition, access to cloud-based back-end service platform can be considered for this use case group.

SA1 has considered both LTE and NR for the candidates of 3GPP RATs in supporting eV2X.

The SA2 study has been progressed to identify and evaluate potential architecture enhancements of EPS and 5G System design needed to support advanced V2X services identified in TR 22.886, based on vehicular services requirements defined in SA1 V2X (TS 22.185) and eV2X (TS 22.186) and determine which of the solutions can proceed to normative specifications. SA2 has completed the study and started the normative work on the architecture solutions for V2X.

TSG RAN has already agreed in TR 38.913 that it is not intended for NR V2X to replace the services offered by LTE V2X. Instead, the NR V2X shall complement LTE V2X for advanced V2X services and support interworking with LTE V2X. At least from 3GPP RAN technology development standpoint, the focus and scope of NR V2X is to target advanced V2X use cases. However, this does not imply that NR V2X cannot support basic safety use cases. It is clearly up to the regional regulators and the stakeholders involved (i.e. Car OEMs and automotive ecosystem in general) to decide on the technology of choice for any service and any use case.

NR V2X is destined as 3GPP V2X phase 3 and would support advanced V2X services beyond services supported in LTE Rel-15 V2X.  The advanced V2X services would require enhanced NR system and new NR sidelink to meet the stringent requirements.  NR V2X system is expected have a flexible design in support of services with low latency and high reliability requirements.  NR system also expects to have higher system capacity and better coverage. The flexibility of NR sidelink framework would allow easy extension of NR system to support the future development of further advanced V2X services and other services. RAN WGs have conducted the feasibility study on NR V2X, and it was concluded in TR 38.885 that it is feasible to support advanced V2X services using the technical solutions identified during the study. It is noted that the study on the remote driving use case was conducted in TR 38.824. Meanwhile, it was noted that solutions defined for V2X can also be used for public safety when the service requirement can be met.

4.2	Objective

The objective of this work item is to specify radio solutions that are necessary for NR to support advanced V2X services (except the remote driving use case which was studied in TR 38.824) based on the study outcome captured in TR 38.885. RAN4 should focus on the RAN WG4 work scope in the approved WID [1] as below

1. NR sidelink: Specify NR sidelink solutions necessary to support sidelink unicast, sidelink groupcast, and sidelink broadcast for V2X services, considering in-network coverage, out-of-network coverage, and partial network coverage.

- Solutions for ‘not co-channel’ in-device coexistence between LTE and NR sidelinks

  - TDM-based solutions as per the study outcome [RAN1, RAN2, RAN4]

  - FDM-based solutions with static power allocation as per the study outcome [RAN4]

  - This will not consider the case where LTE and NR sidelinks are in the same frequency band.

  - No impact to LTE specifications at least from RAN1 and RAN2 perspective.

- UE Tx and Rx RF requirement [RAN4]

  - This requirement should ensure

  - coexistence between sidelink and Uu interface in the same and adjacent channels in licensed spectrum

  - coexistence with other V2X technologies in the adjacent channel in ITS spectrum in 5.9 GHz, without assuming that 5.9 GHz spectrum will be universally available nor that it will be universally available in sufficient quantity to support NR V2X advanced use cases

- RRM core requirement [RAN4]

The solutions should cover both the operating scenario where the carrier(s) is/are dedicated to V2X services and the operating scenario where the carrier(s) is/are licensed spectrum and also used for NR Uu/LTE Uu operation.

NR sidelink design starts with frequencies in FR1, and NR sidelink in FR2 is supported by applying the design for FR1 and PT-RS to the numerologies agreed for FR2. No FR2 specific optimization is supported in this WI except PT-RS. No beam management is supported in this work.

For the scenarios of NR sidelink carrier, this work will consider a single carrier for the NR sidelink transmission and reception.

In this work, 2Rx antennas as well as 4Rx antennas are supported. The full range of speeds defined in SA1 (TS 22.186) needs to be supported in FR1.

It is assumed that any co-channel coexistence requirements and mechanisms of NR sidelink with non-3GPP technologies will not be defined by 3GPP.

5	Evaluation of adjacent channel co-existence

5.1	Adjacent channel coexistence Scenarios

In this section, the adjacent channel (UE-to-UE) coexistence was studied for NR V2X services.  The operating scenarios include both cases where the carrier is dedicated ITS spectrum (e.g. 5.9GHz) and case where the carrier is licensed spectrum.  For the ITS spectrum (e.g. 5.9GHz), the coexistence evaluation include LTE based V2V operation and/or DSRC/IEEE 802.11p operation on the adjacent carrier of NR V2X.  For the licensed carrier, the coexistence evaluation include LTE or NR Uu.

5.1.1	Coexistence scenarios

The evaluation scenarios for adjacent channel coexistence for NR V2X sidelink is shown in Table 5.1.1-1

Table 5.1.1-1: 5G V2X service coexistence scenarios in adjacent channel

| 5G V2X operating frequency |  | Deployment scenarios (Aggressor-to-Victim) |
| --- | --- | --- |
| FR1 | V2X service at ITS spectrum (sidelink: 5.9GHz) | - Case1: NR V2X UE-to-DSRC UE - Case2: DSRC UE-to-NR V2X UE - Case3: NR V2X UE-to-LTE V2X UE - Case4: LTE V2X UE-to-NR V2X UE |
|  | V2X service at licensed bands (TDD: 3.5GHz) | - Case5: NR V2X UE-to-NR Uu BS - Case6: NR Uu UE-to-NR V2X UE |
|  | V2X service at licensed bands (FDD: 2GHz) | - Case7: NR V2X UE-to-LTE Uu BS - Case8: LTE Uu UE-to-NR V2X UE |
| FR2 | V2X service at licensed bands | - Case9: NR V2X UE-to-NR Uu BS - Case10: NR Uu UE-to-NR V2X UE |

Basic simulation parameters are below

- Deployment scenarios: Urban Manhattan grid model, Highway (optional)

- Simulation Block Size :

  - Urban : Manhattan grid model: 3*433m, 3*250m

- Traffic model:

  - Dependent on UE velocity (LTE ref.:36.785, NR ref.: TR37.885)

    - For the NR traffic, medium intensity aperiodic traffic model will be used.

    - LTE traffic model

      - Option1: UE activation rate: 1%

      - Option2: 15km : 0.1% or 60km: 0.33%

      - Other options are not precluded

- RAN1 dependent parameter

  - Power control scheme, RAN4 can revisit the PC according to RAN1 decision, but following is initial

    - In licensed spectrum, OLPC in TR36.786 is used

    - In ITS spectrum, worst case of no power control is used

  - For licensed band, NR SL operation in Uplink band in FDD, UL opportunity in TDD is considered.

  - For SINR calculation in partial overlapping between aggressor and victim, worst case SINR should be considered.

The details of the deployment scenarios are presented in the following clauses.

5.2	Coexistence simulation assumptions

5.2.1	Coexistence simulation parameters in ITS spectrum

5.2.1.1 Layout model

Figure 5.2.1.1-1 Network layout model for NR V2X coexistence simulation in ITS spectrumTable 5.2.1-1: Simulation layout model in ITS spectrum

| Parameter | Value |
| --- | --- |
| carrier frequency | 5.9GHz |
| Deployment scenarios | Urban Mahathan grid model |
| Simulation Block Size | 3*433m, 3*250m |
| Velocity of vehicles | 15km/h or 60km/h Note1: Fixed location will be considered for adjacent coexistence evaluation. Note2: Velocity only used to decide the UE density. |
| Pathloss model for - DSRC UE-to-DSRC UE - LTE V2X UE-to-LTE V2X UE - NR V2X UE-to-DSRC UE - DSRC UE-to-NR V2X UE - NR V2X UE-to-LTE V2X UE - LTE V2X UE-to-NR V2X UE | LOS/NLOS/NLOSv; Pathloss [dB]; Shadow fading std [dB]. LOS, NLOSv; For Highway case, PL = 32.4 + 20 log10(d3D) + 20 log10(fc) For Urban case, PL= 38.77 + 16.7 log10(d3D) + 18.2 log10(fc); σSF = 3. NLOS; PL= 36.85 + 30 log10(d3D) + 18.9 log10(fc); σSF = 4. Note 1: fc denotes the center frequency in GHz and d3D denotes the Euclidean distance between TX and RX in 3D space in meters. Note 2: The model for spatial correlation of shadow fading defined in TR 36.885 applies.. |
| Dropping vehicles (based on clause 6.1.2 from TR 37.885) | Vehicles are dropped according to the following process: - The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec}. - All the vehicles in the same lane have the same speed. - Vehicle type distribution is not dependent of the lane. |
| Total number of vehicles | It depends on vehicle UE density and block size. |
| vehicle UE activation rate for aggressor and victim | 1% of total number of UEs |
| vehicle type (based on clause 6.1.2 from TR 37.885) | - Type 2 (passenger vehicle with higher antenna position): length 5 meters, width 2.0 meters, height 1.6 meters, antenna height 1.6 meters |

5.2.1.2 Simulation parameters

Table 5.2.1.2-1: Simulation parameters in ITS spectrum

| Parameter | Value |  |  |
| --- | --- | --- | --- |
|  | DSRC UE | LTE V2X UE | NR V2X UE |
| Tx power | 23dBm | 23dBm | 23dBm |
| Channel Bandwidth | 10MHz | 20MHz | 40MHz |
| Packet size | 1) 190 byte packet 2) 300 byte packet | 1)14 PRB (190 byte packet) 2)20 PRB (300 byte packet) | 1)[70] PRB (1100 byte packet) for 15kHz SCS 2)Other options are not precluded |
| Traffic model | 1 transmission every 100ms - 100ms message generation period - Time instance of message generation is randomized among vehicles |  |  |
| Noise figure | 10dB | 9dB | 9dB |
| Antenna pattern | Omni-directional with gain of 0 dBi |  |  |
| Sidelink Power control | In ITS spectrum, worst case of no power control is used |  |  |
| SINR-to-BLER mapping | As per link level performance model in TR 36.785 Table A-4 for 5.9GHz | As per link level performance model in TR 36.785 Table A-3 for 5.9GHz | Reuse LTE V2X link level performance for 300 Bytes in TR 36.785Table A-3 for 5.9GHz. Will be updated once link level design is finalised in RAN1. |

Table 5.2.1.2-2: Additional simulation assumptions for DSRC UE

| Parameter | Value |
| --- | --- |
| DSRC MCS | QPSK with r=1/2 Transmit duration of 341us (190bytes) and 488 (300bytes) without header |
| DSRC MAC | Coordination : DL+UL coexistence evaluation : EDCA |
|  | Detection : Energy detection & preamble detection |
| DSRC Slot time | 13us (Note specified for 10MHz [802.11 -2012]) |
| DSRC DIFS | 58us (SIFS + 2*slot_time) |
| DSRC SIFS | 32us |
| DSRC Physical header size | 40us |
| DSRC Threshold triggering physical header decoding | -98dBm |
| DSRC CCA-CS | -85dBm |
| DSRC CCA-ED | -65dBm |
| OFDM symbol duration | 8us |

5.2.1.3 ACLR and ACS

- Option 1 (as baseline) : 1 step ACLR/ACS model

Table 5.2.1.3-1: ACLR and ACS in ITS spectrum

| Parameter | Value |  |  |
| --- | --- | --- | --- |
|  | DSRC UE | LTE V2X UE | NR V2X UE |
| ACLR | 26dB | 30dB | 30 + X dB |
| ACS | Use three candidate ACS level for DSRC : - 22/25/29dB | 27dB | 24+ X dB |

Details for modelling:

- Option 2: 2 steps ACLR/ACS model.

  - ACLR:

    - If the frequency offset between transmitting RB (aggressor) and receiving RB (victim) is smaller than the transmitter (aggressor) allocated transmission bandwidth bandwidth then ACLR1 applies.

    - If the frequency offset between transmitting RB (aggressor) and receiving RB (victim) is larger than the transmitter (aggressor) allocated transmission bandwidth bandwidth then ACLR2 applies

  - ACS

    - If the frequency offset between transmitting RB (aggressor) and receiving channel edge (victim) is smaller than 10MHz then ACS1 applies.

    - If the frequency offset between transmitting RB (aggressor) and receiving channel edge (victim) is larger than 10MHz then ACS2 applies.

NOTE: 10MHz offset velue is derived from LTE V2X definition of ACS and inband blocking requirement, as will as DSRC adjacent and alternate adjacent channel rejection requirement definition

Table 5.2.1.3-2: ACLR1/2 and ACS1/2 in ITS spectrum

| Parameter | Value |  |  |
| --- | --- | --- | --- |
|  | DSRC UE | LTE V2X UE | NR V2X UE |
| ACLR1 | 26dB | 30dB | 30 + X dB |
| ACLR2 | [39]dB | [43]dB | [43]dB |
| ACS1 | Use three candidate ACS level for DSRC : - 22/25/29dB | 27dB | 24+ X dB |
| ACS2 | ACS1 + 16 (NOTE 1) | 43dB (NOTE 2) | [40]dB + X(NOTE 3) |
| NOTE 1: following definition of alternate adjacent channel rejection requirement of DSRC [EN 302 571] NOTE 2: following definition of in band blocking requirement, case 1 of LTE CV2X [TS 36.101] NOTE 3: following definition of in band blocking requirement, case 2 of NR and apply same tightening as for LTE V2X. |  |  |  |

5.2.2	Coexistence simulation parameters in licensed spectrum for FR1

5.2.2.1 Network layout model

Figure 5.2.2.1-1 Network layout model for NR V2X coexistence simulation for case7 and case8

Table 5.2.2-1: Simulation network layout model for case7 and case8

| Parameter | Value |
| --- | --- |
| carrier frequency | 2GHz |
| The first layout for LTE WAN | layer with 7 hexagonal cell with wrap around |
| Inter-BS distance | 500m |
| LTE WAN UL scheduler algorithm | Round robin with full buffer |
| RBs allocated per active WAN UE | For LTE UE-to-LTE Uu BS: 32 PRBs |
| Number of active WAN UEs | 3UEs |
| Number of LTE WAN UEs | 20UEs/Cell, 100% outdoor (UEs are dropped with the equal inter-UE distance in the middle of the sidewalk) |
| Minimum coupling loss (for V2X/WAN UEs from BS) | As per clause 4.5.1 in TR 36.942: MCL : 70dB for Urban area |
| Pathloss model for - LTE UE-to-LTE Uu BS - NR V2X UE-to-LTE Uu BS | Follow TR 37.885 for FR1 FR1. LOS; NLOS. Urban: TR 38.901 UMa LOS Highway: TR 38.901 RMa LOS; Urban: TR 38.901 UMa NLOS Highway: N/A. |
| The second layout for NR V2X | Urban Mahathan grid model |
| Simulation Block Size | 433m, 250m |
| Velocity of vehicles | 15km/h or 60km/h Note1: Fixed location will be considered for adjacent coexistence evaluation. Note2: Velocity only used to decide the UE density. |
| Pathloss model for - LTE Uu UE-to-NR V2X UE | LOS/NLOS/NLOSv; Pathloss [dB]; Shadow fading std [dB]. LOS, NLOSv; For Highway case, PL = 32.4 + 20 log10(d3D) + 20 log10(fc) For Urban case, PL= 38.77 + 16.7 log10(d3D) + 18.2 log10(fc); σSF = 3. NLOS; PL= 36.85 + 30 log10(d3D) + 18.9 log10(fc); σSF = 4. Note 1: fc denotes the center frequency in GHz and d3D denotes the Euclidean distance between TX and RX in 3D space in meters. Note 2: The model for spatial correlation of shadow fading defined in TR 36.885 applies.. |
| Dropping vehicles (based on clause 6.1.2 from TR 37.885) | Vehicles are dropped according to the following process: - The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec}. - All the vehicles in the same lane have the same speed. - Vehicle type distribution is not dependent of the lane. |
| Total number of vehicles | It depends on UE density and block size. All vehicles are NR V2X UEs. |
| Vehicle UE activation rate | 1% of total number of vehicles |
| vehicle type (based on clause 6.1.2 from TR 37.885) | - Type 2 (passenger vehicle with higher antenna position): length 5 meters, width 2.0 meters, height 1.6 meters, antenna height 1.6 meters |

5.2.2.2 Simulation parameters

Table 5.2.2.2-1: Simulation parameters for case7 and case8

| Parameter | Value |  |  |
| --- | --- | --- | --- |
|  | LTE UE | LTE BS | NR V2X UE |
| Max Tx power | 23dBm | NA | 23dBm |
| Channel Bandwidth | 10MHz | 10MHz | 40MHz |
| Packet size | 1)[16] PRB for 15kHz SCS 2)Other options are not precluded |  | 1)[70] PRB (1100 byte packet) for 15kHz SCS 2)Other options are not precluded |
| Traffic model | Full buffer | Full buffer | Reference table 5.2.1.2-1 |
| Noise figure | NA | 5dB | 9dB |
| Antenna pattern | Omni-directional with gain of 0 dBi | From clause 4.2.1.1 of TR 36.942 degrees, and | Omni-directional with gain of 0 dBi |
| SINR-to-BLER mapping for NR V2X | NA | NA | reuse LTE V2X link level performance in TR 36.785 Table A-2 for 2GHz |
| SINR-to-rate mapping for LTE | NA | As per link level performance model in TR 36.942 (Table A.2) | NA |

5.2.2.3 ACLR and ACS

Table 5.2.2.3-1: ACLR and ACS for case7 and case8

| Parameter | Value |  |  |
| --- | --- | --- | --- |
|  | LTE UE | LTE BS | NR V2X UE |
| ACLR | 30 dB | NA | 30 + X dB |
| ACS | NA | 46 dB | 24+ X dB |

5.2.2.4 Power control

For LTE WAN UE transmit power control, the following power control equation shall be used for the initial uplink coexistence simulations:

where Pmax is the maximum transmit power, Rmin is the minimum power reduction ratio to prevent UEs with good channels to transmit at very low power level, CL is the path coupling loss defined as max{path loss-G_Tx-G_Rx, MCL}, where path loss is propagation loss plus shadowfading, G_TX is the transmitter antenna gain in the direction of the receiver, G_RX is the receiver antenna gain in the direction of the transmitter and CLx-ile is the x-percentile CL value. With this power control equation, the x percent of UEs that have the highest coupling loss will transmit at Pmax.  Finally, 0<<=1 is the balancing factor for UEs with bad channel and UEs with good channel:

As per PC set 1 and PC set 2 of TR 36.942

- Note that power control algorithm parameters (PodBm, CLxile) should be optimized for network layouts being simulated. For simplicity, power control algorithm parameters are reused in section 5.1.1.6 in TR 36.942 for all network layouts

- Rmin = -63dB for 23dBm, -66dB for 26dBm

- CLxile = 112 - 10*log10(CBW / 10) for PC set 1

| PC Set | Gamma | CLxile (dBm) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 10MHz | 20MHz | 40MHz |
| 1 | 1 | 112 | 109 | 106 |
| 2 | 0.8 | 129 | FFS | FFS |

For NR V2X UE transmit power control, OLPC in TR36.786 is reused.

CLxile = 112 - 10*log10(CBW / 10) for PC set 1

5.2.3	Coexistence simulation parameters in licensed spectrum for FR2

5.2.3.1 Network layout model

Figure 5.2.3.1-1 Network layout model for NR V2X coexistence simulation for case5 and case6

Table 5.2.3-1: Simulation network layout model for case5 and case6

| Parameter | Value |
| --- | --- |
| carrier frequency | 3.5GHz |
| The first layout for NR WAN | layer with 7 hexagonal cell with wrap around |
| Inter-BS distance | 500m |
| NR WAN UL scheduler algorithm | Round robin with full buffer |
| RBs allocated per active WAN UE | For NR UE-to-NR Uu BS: 64 PRBs |
| Number of active WAN UEs | 3UEs |
| Number of NR WAN UEs | 20UEs/Cell, 100% outdoor (UEs are dropped with the equal inter-UE distance in the middle of the sidewalk) |
| Minimum coupling loss (for V2X/WAN UEs from BS) | As per clause 4.5.1 in TR 36.942: MCL : 70dB for Urban area |
| Pathloss model for - NR UE-to-NR Uu BS - NR V2X UE-to-NR Uu BS | Follow TR 37.885 for FR1 FR1. LOS; NLOS. Urban: TR 38.901 UMa LOS Highway: TR 38.901 RMa LOS; Urban: TR 38.901 UMa NLOS Highway: N/A. |
| The second layout for NR V2X | Urban Mahathan grid model |
| Simulation Block Size | 433m, 250m |
| Velocity of vehicles | 15km/h or 60km/h Note1: Fixed location will be considered for adjacent coexistence evaluation. Note2: Velocity only used to decide the UE density. |
| Pathloss model for - NR Uu UE-to-NR V2X UE | LOS/NLOS/NLOSv; Pathloss [dB]; Shadow fading std [dB]. LOS, NLOSv; For Highway case, PL = 32.4 + 20 log10(d3D) + 20 log10(fc) For Urban case, PL= 38.77 + 16.7 log10(d3D) + 18.2 log10(fc); σSF = 3. NLOS; PL= 36.85 + 30 log10(d3D) + 18.9 log10(fc); σSF = 4. Note 1: fc denotes the center frequency in GHz and d3D denotes the Euclidean distance between TX and RX in 3D space in meters. Note 2: The model for spatial correlation of shadow fading defined in TR 36.885 applies.. |
| Dropping vehicles (based on clause 6.1.2 from TR 37.885) | Vehicles are dropped according to the following process: - The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec}. - All the vehicles in the same lane have the same speed. - Vehicle type distribution is not dependent of the lane. |
| Total number of vehicles | It depends on UE density and block size. All vehicles are NR V2X UEs. |
| Vehicle UE activation rate | 1% of total number of vehicles |
| vehicle type (based on clause 6.1.2 from TR 37.885) | - Type 2 (passenger vehicle with higher antenna position): length 5 meters, width 2.0 meters, height 1.6 meters, antenna height 1.6 meters |

5.2.3.2 Simulation parameters

Table 5.2.3.2-1: Simulation parameters for case5 and case6

| Parameter | Value |  |  |
| --- | --- | --- | --- |
|  | NR UE | NR BS | NR V2X UE |
| Max Tx power | 23dBm | NA | 23dBm |
| Channel Bandwidth | 40MHz | 40MHz | 40MHz |
| Packet size | 1)[72] PRB for 15kHz SCS 2)Other options are not precluded |  | 1)[70] PRB (1100 byte packet) for 15kHz SCS 2)Other options are not precluded |
| Traffic model | Full buffer | Full buffer | Reference table 5.2.1.2-1 |
| Noise figure | NA | 5dB | 9dB |
| Antenna pattern | Omni-directional with gain of 0 dBi | Antenna pattern for FR1 Macro BS from TR 38.828 | Omni-directional with gain of 0 dBi |
| SINR-to-BLER mapping for NR V2X | NA | NA | FFS or reuse LTE V2X link level performance in TR 36.785 Table A-2 for 2GHz |
| SINR-to-rate mapping for NR | NA | As per link level performance model in TR 36.942 (Table A.2). α, attenuation = 0.4, SNIRMIN, dB = -10, SNIRMAX, dB = 22 (subclause 5.2.3.6 from TR 38.828). | NA |

5.2.3.3 ACLR and ACS

Table 5.2.3.3-1: ACLR and ACS for case5 and case6

| Parameter | Value |  |  |
| --- | --- | --- | --- |
|  | NR UE | NR BS | NR V2X UE |
| ACLR | 30 dB | NA | 30 + X dB |
| ACS | NA | 46 dB | 33+ X dB |

5.2.3.4 Power control

For downlink scenario, no power control scheme is applied.

For NR WAN UE uplink scenario transmit power control, the following power control equation shall be used for the initial uplink coexistence simulations:

where Pmax is the maximum transmit power, Rmin is the minimum power reduction ratio to prevent UEs with good channels to transmit at very low power level, CL is the path coupling loss defined as max{path loss-G_Tx-G_Rx, MCL}, where path loss is propagation loss plus shadowfading, G_TX is the transmitter antenna gain in the direction of the receiver, G_RX is the receiver antenna gain in the direction of the transmitter and CLx-ile is the x-percentile CL value. With this power control equation, the x percent of UEs that have the highest coupling loss will transmit at Pmax.  Finally, 0<<=1 is the balancing factor for UEs with bad channel and UEs with good channel:

-	CLx-ile = 88 + 10*log10(200/X), where X is UL transmission BW (MHz)

-	γ = 1

-	Rmin = -60dB for 23dBm

For NR V2X UE transmit power control, OLPC in TR36.786 is reused

CLxile = 112 - 10*log10(CBW / 10) for PC set 1

5.2.4	PRR performance metrics for NR V2X coexistence simulation

For evaluation of co-existence performance for NR V2X, the Packet Reception Ratio (PRR) is agreed in RAN4 to be used as performance metric.

In TR 37.885, for one Tx packet, the PRR is calculated by X/Y, where Y is the number of UE/vehicles that located in the range (a, b) from the TX, and X is the number of UE/vehicles with successful reception among Y.

-	CDF of PRR is used in evaluation with with a = 0, b = baseline of 320 meters for freeway and 150 meters for urban. Optionally, b = 50 meters for urban with 15 km/h vehicle speed.

-	Average PRR, calculated as (X1+X2+X3….+Xn)/(Y1+Y2+Y3…+Yn) where n denotes the number of generated messages in simulation. with a = i*20 meters, b = (i+1)*20 meters for i=0, 1, …, 25

In RAN4 co-existence study for NR V2X, only urban cases with 15 km/h and 60 km/h are considered.

To align with LTE V2X’s performance metric, average PRR at 50m for 15 km/h and average PRR at 150m for 60 km/h are chosen as the evaluation criteria.

5.3	Coexistence simulation results

5.3.1	Coexistence simulation results in ITS spectrum

5.3.2	Coexistence simulation results in licensed spectrum for FR1

5.3.3	Coexistence simulation results in licensed spectrum for FR2

6	Evaluation of In-device Coexistence

6.1 In-device coexistence scenarios

7	Operating bands and channel bandwidth

7.1	Operating bands

7.1.1	Operating bands in FR1

NR V2X communication is designed to operate in the operating bands in FR1 defined in Table 7.1.1-1.

Table 7.1.1-1 NR V2X operating bands in FR1

| NR V2X Operating Band | Sidelink (SL) Transmission operating band |  |  | Sidelink (SL) Reception operating band |  |  | Duplex Mode | Interface |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |
| n47 | 5855 MHz | - | 5925 MHz | 5855 MHz | - | 5925 MHz | TDD | PC5 |

7.1.2	Operating bands in FR2

7.2	Channel bandwidth

7.2.1	Channel bandwidth in FR1

The NR V2X Communication channel bandwidths and operating bands are shown in Table 7.2.1-1. The same (symmetrical) channel bandwidth is specified for both the TX and RX path.

Table 7.2.1-1 NR V2X Communication channel bandwidth

|  | NR V2X band / V2X channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR V2X Operating Band | 10 MHz | 20 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| 47 | Yes | Yes |  | Yes |  |  |  |  |  |

7.2.2	Channel bandwidth in FR2

8	Transmitter characteristics

8.1	NR V2X UE Tx requirements in FR1

8.2	NR V2X UE Tx requirements in FR2

9	Receiver characteristics

9.1	NR V2X UE Rx requirements in FR1

9.2	NR V2X UE Rx requirements in FR2

10	Tx/Rx characteristics of concurrent operation

10.1	NR V2X SL operation in 5.9GHz or other potential ITS spectrum and LTE/NR Uu operation in licensed band for FR1

10.2	NR V2X SL operation and LTE/NR Uu operation both in licensed band for FR1

10.3	NR V2X SL operation in 5.9GHz or other potential ITS spectrum and NR Uu operation in licensed bands for FR2

10.4	NR V2X SL operation and NR Uu operation both in licensed bands for FR2

11	Conclusion and recommendations

Annex A:

Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-04 | RAN4 #90BIS | R4-1905096 |  |  |  | TR38.886 v0.0.1: TR skeleton for NR V2X service with SL operation | 0.0.1 |
| 2019-05 | RAN4 #91 | R4-1906061 |  |  |  | TR38.886 v0.1.0: TR update for NR V2X service with SL operation | 0.1.0 |
|  |  |  |  |  |  | Approved TP lists in RAN4 #91 meeting - R4-1906851 TP on In-device Coexistence Scenarios - R4-1907607 TP on Editorial Corrections to TR38.886 |  |
| 2019-08 | RAN4 #92 | R4-1908696 |  |  |  | TR38.886 v0.2.0: TR update for NR V2X service with SL operation | 0.2.0 |
|  |  |  |  |  |  | Approved TP lists in RAN4 #91 meeting - R4-1908378 TP for band and bandwidth for NR V2X - R4-1910393 TP for band and bandwidth for NR V2X - R4-1910400 TP on adjacent channel co-existence scenarios and parameters for NR V2X service on FR1 - R4-1908351 TP on Adjacent Channel Coexistence - R4-1908973 TP on PRR performance metrics for NR V2X coexistence simulation |  |
| 2019-10 | RAN4 #82BIS | R4-191xxxxx |  |  |  | TR38.886 v0.3.0: TR update for NR V2X service with SL operation | 0.3.0 |