| 3GPP TR 38.891 V0.0.1 (2022-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; User Equipment (UE) Further enhancements of NR RF requirements for frequency range 2 (FR2) for Rel-18 (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Background	7

5	Study for FR2 UL 256QAM	7

5.1	General	7

5.2	Simulation	7

5.2.1	System level simulation	7

5.2.2	Link level simulation	7

5.2.3	MPR simulation	7

5.3	Implementation impact for UE	8

5.4	Specification impact	8

5.5	Conclusion	8

6	Study for beam correspondence	8

6.1	General	8

6.2	Implementation impact to UE	8

6.3	Specification Impact	8

6.3.1	RF	8

6.3.2	RRM	8

6.4	Testing impact	8

6.5	Conclusion	8

Annex <A> (informative): Change history	9

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In drafting the TS/TR, pay particular attention to the use of modal auxiliary verbs! TRs shall not contain any normative provisions.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document captures finding of the study on FR2 UL 256QAM and beam correspondence for RRC_INACTIVE and initial access. The purpose of this TR is to study the performance gain and implementation aspects for FR2 UL 256QAM and to ensure good UE RACH msg1 performance and UL coverage in FR2 deployments by specifying the beam correspondence requirements for RRC_INACTIVE and initial access as defined in Work Item “NR RF requirements enhancement for frequency range 2 (FR2), Phase 3” [2].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-220967, “new WID: NR RF requirements enhancement for frequency range 2 (FR2), Phase 3”, Nokia, Xiaomi

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ACRONYM>	<Explanation>

# 4	Background

At the 3GPP RAN #95 meeting, the Work Item on “NR RF requirements enhancement for frequency range 2 (FR2), Phase 3” was approved for Rel-18. The objectives of the core part in the Work Item are as follows:

UL 256QAM

- Investigate and enable UL 256QAM for FR2-1 [RAN4]

  - Study the gain, operating SNR, phase noise model and implementation aspects

  - Specify the UE RF requirements

  - First priority: Targeted power classes are PC1, PC2 and PC5

  - Second priority: Targeted power class is PC3

Beam correspondence requirements for RRC_INACTIVE and initial access

- Specify UE beam correspondence requirements for initial access and RRC_INACTIVE state, for SSB-based beam correspondence without UL beam sweeping [RAN4 RF]

- For RRC_INACTIVE specify at least requirements for Random Access SDT and Configured Grant SDT

  - Requirements for other transmission within RRC_INACTIVE state are not precluded.

- For initial access, specify requirements and verification of beam correspondence requirements based on msg1 spherical coverage (at least)

Study the potential impact on testability aspects (i.e., test time).

This TR aims to study the objectives and further capture the impact on the specification if applicable.

# 5	Study for FR2 UL 256QAM

Editor’s note: This clause collects the simulation results and implementation aspects for FR2 UL 256QAM study for Rel-18. If necessary, new agenda can be assigned.

## 5.1	General

## 5.2	Simulation

### 5.2.1	System level simulation

### 5.2.2	Link level simulation

### 5.2.3	MPR simulation

## 5.3	Implementation impact for UE

## 5.4	Specification impact

## 5.5	Conclusion

# 6	Study for beam correspondence

## 6.1	General

## 6.2	Implementation impact to UE

## 6.3	Specification Impact

### 6.3.1	RF

### 6.3.2	RRM

## 6.4	Testing impact

## 6.5	Conclusion

Editor’s note:

# Annex <A> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-08 | RAN4 #104e | R4-22xxxxx |  |  |  | TR skeleton | 0.0.1 |

Change history of this template:

| 2001-07 | Copyright date changed to 2001; space character added before TTC in copyright notification; space character before first reference deleted. | 1.3.3 |
| --- | --- | --- |
| 2002-01 | Copyright date changed to 2002. | 1.3.4 |
| 2002-07 | Extra Releases added to title area. | 1.3.5 |
| 2002-12 | "TM" added to 3GPP logo. | 1.3.6 |
| 2003-02 | Copyright date changed to 2003. | 1.3.7 |
| 2003-12 | Copyright date changed to 2004. Chinese OP changed from CWTS to CCSA | 14.0 |
| 2004-04 | North American OP changed from T1 to ATIS | 1.5.0 |
| 2005-11 | Stock text of clause 3 includes reference to 21.905. | 1.6.0 |
| 2005-11 | Caters for new TSG structure. Minor corrections. | 1.6.1 |
| 2006-01 | Revision marks removed. | 1.6.2 |
| 2008-11 | LTE logo line added, © date changed to 2008, guidance on keywords modified; acknowledgement of trade marks; sundry editorial corrections and cosmetic improvements | 1.7.0 |
| 2010-02 | 3GPP logo changed for cleaner version, with tag line; LTE-Advanced logo line added; © date changed to 2010; editorial change to cover page footnote text; trade marks acknowledgement text modified; additional Releases added on cover page; proforma copyright release text block modified | 1.8.0 |
| 2010-02 | Smaller 3GPP logo file used. | 1.8.1 |
| 2010-07 | Guidance note concerning use of LTE-Advanced logo added. | 1.8.2 |
| 2011-04-01 | Guidance of use of logos on cover page modified; copyright year modified. | 1.8.3 |
| 2013-05-15 | Changed File Properties to MCC macro default. Removed R99, added Rel-12/13. Modified Copyright year. Guidance on annex X Change history. | 1.8.4 |
| 2014-10-27 | Updated Release selection on cover. In clause 3, added "3GPP" to TR 21.905. | 1.8.5 |
| 2015-01-06 | New Organizational Partner TSDSI added to copyright block. Old Releases removed. | 1.9.0 |
| 2015-12-03 | Provision for LTE Advanced Pro logo Update copyright year to 2016 | 1.10.0 |
| 2016-03-08 | Standarization of the layout of the Change History table in the last annex.(Unreleased) | 1.11.0 |
| 2016-06-15 | Minor adjustment to Change History table heading | 1.11.1 |
| 2017-03-13 | Adds option for 5G logo on cover | 1.12.0 |
| 2017-05-03 | Smaller 5G logo to reduce file size | 1.12.1 |
| 2019-02-25 | Replacement of frames on cover pages by in-line text. Clarification of help text on when to use 5G logo. Removal of defunct keywords frame on page 2. Add Rel-16, Rel-17 options, eliminated earlier, frozen, Releases (cover page, below title) Corrections to some guidance text, addition of guidance text concerning automatic page headers under Word 2016 ff. Use of modal auxiliary verbs added to Foreword. More explicit guidance on Bibliography and Index annexes. Converted to .docx format. | 1.13.0 |