| 3GPP TR 38.891 V0.5.0 (2023-05) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; User Equipment (UE) Further enhancements of NR RF requirements for frequency range 2 (FR2) for Rel-18 (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Background	7

5	Study for FR2 UL 256QAM	7

5.1	General	7

5.2	Simulation	7

5.2.1	System level simulation	7

5.2.2	Link level simulation	7

5.2.2.1               Simulation assumptions	8

5.2.3	MPR simulation	8

5.3	Implementation impact for UE	9

5.4	Specification impact	9

5.5	Conclusion	9

6	Study for beam correspondence	9

6.1	General	9

6.2	Implementation impact to UE	9

6.3	Specification Impact	9

6.3.1	RF	9

6.3.2	RRM	9

6.4	Testing impact	9

6.5	Conclusion	9

Annex <A> (informative): Change history	10

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In drafting the TS/TR, pay particular attention to the use of modal auxiliary verbs! TRs shall not contain any normative provisions.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document captures finding of the study on FR2 UL 256QAM and beam correspondence for RRC_INACTIVE and initial access. The purpose of this TR is to study the performance gain and implementation aspects for FR2 UL 256QAM and to ensure good UE RACH msg1 performance and UL coverage in FR2 deployments by specifying the beam correspondence requirements for RRC_INACTIVE and initial access as defined in Work Item “NR RF requirements enhancement for frequency range 2 (FR2), Phase 3” [2].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-220967, “new WID: NR RF requirements enhancement for frequency range 2 (FR2), Phase 3”, Nokia, Xiaomi

[3]                       3GPP TR 38.803: "Study on new radio access technology: RF and co-existence aspects".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ACRONYM>	<Explanation>

# 4	Background

At the 3GPP RAN #95 meeting, the Work Item on “NR RF requirements enhancement for frequency range 2 (FR2), Phase 3” was approved for Rel-18. The objectives of the core part in the Work Item are as follows:

UL 256QAM

- Investigate and enable UL 256QAM for FR2-1 [RAN4]

  - Study the gain, operating SNR, phase noise model and implementation aspects

  - Specify the UE RF requirements

  - First priority: Targeted power classes are PC1, PC2 and PC5

  - Second priority: Targeted power class is PC3

Beam correspondence requirements for RRC_INACTIVE and initial access

- Specify UE beam correspondence requirements for initial access and RRC_INACTIVE state, for SSB-based beam correspondence without UL beam sweeping [RAN4 RF]

- For RRC_INACTIVE specify at least requirements for Random Access SDT and Configured Grant SDT

  - Requirements for other transmission within RRC_INACTIVE state are not precluded.

- For initial access, specify requirements and verification of beam correspondence requirements based on msg1 spherical coverage (at least)

Study the potential impact on testability aspects (i.e., test time).

This TR aims to study the objectives and further capture the impact on the specification if applicable.

# 5	Study for FR2 UL 256QAM

Editor’s note: This clause collects the simulation results and implementation aspects for FR2 UL 256QAM study for Rel-18. If necessary, new agenda can be assigned.

## 5.1	General

## 5.2	Simulation

### 5.2.1	System level simulation

The system level simulation as supplementary for link level simulation is to check whether the UE working on FR2 UL 256QAM can achieve target SNR at BS side and to further confirm FR2 UL 256QAM is feasible.

#### 5.2.1.1   Simulation assumptions

Table 5.2.1.1-1 System level simulation assumptions

| Parameters |  | Urban macro | Indoor |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around | 50m x 120m, 12BSs |
| Inter-site distance |  | 200m (baseline) 300m (optional) | 20m |
| BS antenna height |  | 25 m | 3 m |
| UE location | Outdoor/indoor | Outdoor and indoor | Indoor |
|  | Indoor UE ratio | 20% |  |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | UE antenna height | Same as 3D-Uma in TR 36.873 | 1.5 m |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS – UE distance (2D) |  | 35 m | 0 m |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |
| Pathloss |  | Uma LOS and NLOS in table 5.2.2.1-1 of 38.803 | InH – Office LOS and NLOS in table 5.2.2.1-1 of 38.803 |
| Carrier frequency |  | 29GHz, 39GHz |  |
| BS antenna configuration |  | (Mg, Ng, M, N, P) = (1, 1, 8, 16, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 8 dBi | (Mg, Ng, M, N, P) = (1, 1, 8, 16, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 5 dBi |
| UE antenna configuration |  | First priority: PC1/PC2/PC5: (Mg, Ng, M, N, P) = (1, 1, 4, 4, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 5 dBi Second priority: PC3: (Mg, Ng, M, N, P) = (1, 1, 2, 2, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 5 dBi |  |
| System bandwidth |  | 200MHz |  |
| Target SNR at BS side |  | 28dB for 29GHz 30dB for 39GHz |  |
| UE max output power |  | PC1: 35 dBm/PC2: 23dBm/PC3: 23 dBm/PC5: 23 dBm |  |
| UE Min peak EIRP |  | n257 PC1:40.0 dBm/PC2:29 dBm/PC3:22.4 dBm/PC5: 30dBm |  |
| Power control |  | Power control parameters set to achieve the target SNR at BS side. |  |

#### 5.2.1.2   Simulation results from Nokia

R4-2218327

The UE transmit power and UL SINR CDF at 29 GHz (n257) and 39 GHz (n260) using the agreed parameters in urban macro scenario (with the target SNR at BS side of 28 dB and 25 dB, respectively, at 29 GHz and 39 GHz) are shown in figures 5.2.1.2-1 and 5.2.1.2-2, respectively.

a) PC1 UE with 35 dBm maximum output power

b) PC2/5 UE with 23 dBm maximum output power

Figure 5.2.1.2-1. Urban macro simulation results at 29 GHz (n257)

a) PC1 UE with 35 dBm maximum output power

b) PC2/5 UE with 23 dBm maximum output power

Figure 5.2.1.2-2. Urban macro simulation results at 39 GHz (n260)

It can be observed from figures 5.2.1.2-1 and 5.2.1.2-2 that at both 29 GHz (n257) and 39 GHz (n260):

-	The target SNR at BS side (of 28 dB and 25 dB, respectively, at 29 GHz and 39 GHz) can be achieved by ~70% of PC1 UE and ~60% of PC2/5 UE.

-	~15% of PC1 UE and ~25% of PC2/5 UE are transmitting at maximum output power (of 35 dBm and 23 dBm, respectively, for PC1 UE and PC2/5 UE).

The UE transmit power and UL SINR CDF at 29 GHz (n257) and 39 GHz (n260) using the agreed parameters in indoor scenario (with the target SNR at BS side of 28 dB and 25 dB, respectively, at 29 GHz and 39 GHz) are shown in figures 5.2.1.2-3 and 5.2.1.2-4, respectively.

a) PC1 UE with 35 dBm maximum output power

b) PC2/5 UE with 23 dBm maximum output power

Figure 5.2.1.2-3. Indoor simulation results at 29 GHz (n257)

a) PC1 UE with 35 dBm maximum output power

b) PC2/5 UE with 23 dBm maximum output power

Figure 5.2.1.2-4. Indoor simulation results at 39 GHz (n260)

It can be observed from figures 5.2.1.2-3 and 5.2.1.2-4 that at both 29 GHz (n257) and 39 GHz (n260):

-	The target SNR at BS side (of 28 dB and 25 dB, respectively, at 29 GHz and 39 GHz) can be achieved by ~70% of PC1 UE and PC2/5 UE.

-	Almost none of PC1 UE and PC2/5 UE are transmitting at maximum output power (of 35 dBm and 23 dBm, respectively, for PC1 UE and PC2/5 UE).

R4-2300193

The UE transmit power and UL SINR CDF at 29 GHz (n257) and 39 GHz (n260) using the agreed parameters in urban macro scenario (with the target SNR at BS side of 27 dB at both 29 GHz and 39 GHz) are shown in figures 5.2.1.2-5 and 5.2.1.2-6, respectively.

a) PC1 UE with 35 dBm maximum output power

b) PC2/5 UE with 23 dBm maximum output power

Figure 5.2.1.2-5. Urban macro simulation results at 29 GHz (n257)

a) PC1 UE with 35 dBm maximum output power

b) PC2/5 UE with 23 dBm maximum output power

Figure 5.2.1.2-6. Urban macro simulation results at 39 GHz (n260)

It can be observed from figures 5.2.1.2-5 and 5.2.1.2-6 that at both 29 GHz (n257) and 39 GHz (n260):

-	The target SNR at BS side of 27 dB can be achieved by ~70% of PC1 UE and ~60% of PC2/5 UE.

-	Less than 15% of PC1 UE and less than 30% of PC2/5 UE are transmitting at maximum output power (of 35 dBm and 23 dBm, respectively, for PC1 UE and PC2/5 UE).

The UE transmit power and UL SINR CDF at 29 GHz (n257) and 39 GHz (n260) using the agreed parameters in indoor scenario (with the target SNR at BS side of 27 dB at both 29 GHz and 39 GHz) are shown in figures 5.2.1.2-7 and 5.2.1.2-8, respectively.

a) PC1 UE with 35 dBm maximum output power

b) PC2/5 UE with 23 dBm maximum output power

Figure 5.2.1.2-7. Indoor simulation results at 29 GHz (n257)

a) PC1 UE with 35 dBm maximum output power

b) PC2/5 UE with 23 dBm maximum output power

Figure 5.2.1.2-8. Indoor simulation results at 39 GHz (n260)

Note that the results for PC1 and PC2/5 UE are alike in figures 5.2.1.2-7 and 5.2.1.2-8 as the higher PC1 UE transmit power is not required in indoor scenario. It can be observed from figures 5.2.1.2-7 and 5.2.1.2-8 that at both 29 GHz (n257) and 39 GHz (n260):

-	The target SNR at BS side of 27 dB can be achieved by ~70% of PC1 UE and PC2/5 UE.

-	Almost none of PC1 UE and PC2/5 UE are transmitting at maximum output power (of 35 dBm and 23 dBm, respectively, for PC1 UE and PC2/5 UE).

R4-2307231

The UE transmit power and UL SINR CDF at 39 GHz (n260) using the agreed parameters in urban macro scenario (with the target SNR at BS side of 30 dB) are shown in figure 5.2.1.2-9.

a) PC1 UE with 35 dBm maximum output power

b) PC2/5 UE with 23 dBm maximum output power

Figure 5.2.1.2-9. Urban macro simulation results at 39 GHz (n260)

It can be observed from figure 5.2.1.2-9 that at 39 GHz (n260):

-	The target SNR at BS side of 30 dB can be achieved by ~60% of PC1 UE and ~50% of PC2/5 UE.

-	~18% of PC1 UE and ~32% of PC2/5 UE are transmitting at maximum output power (of 35 dBm and 23 dBm, respectively, for PC1 UE and PC2/5 UE).

The UE transmit power and UL SINR CDF at 39 GHz (n260) using the agreed parameters in indoor scenario (with the target SNR at BS side of 30 dB) are shown in figure 5.2.1.2-10.

a) PC1 UE with 35 dBm maximum output power

b) PC2/5 UE with 23 dBm maximum output power

Figure 5.2.1.2-10. Indoor simulation results at 39 GHz (n260)

It can be observed from figure 5.2.1.2-10 that at 39 GHz (n260):

-	The target SNR at BS side of 30 dB can be achieved by ~60% of PC1 UE and PC2/5 UE.

-	Almost none of PC1 UE and PC2/5 UE are transmitting at maximum output power (of 35 dBm and 23 dBm, respectively, for PC1 UE and PC2/5 UE).

Therefore, from the observations above, it can be concluded:

-	In all simulated scenarios, a significant percentage of PC1/PC2/PC5 UE can achieve the target SNR at BS side above which 256QAM provides throughput gain over 64QAM at both 29 GHz (n257) and 39 GHz (n260).

#### 5.2.1.3   Simulation results from ZTE

R4-2218595

In order to check whether the UE working on FR2 UL 256QAM can achieve target SNR at BS side we provide some system level simulation results with target SNR of 31dB and proposed Min peak EIRP. Figure 5.2.1.3-1~figure 5.2.1.3-4 show the CDF of SINR for PC1, PC2, PC3 and PC5 in band n257.

Figure 5.2.1.3-1, CDF of SINR for PC1 under Urban Macro with Fc = 29GHz

Figure 5.2.1.3-2, CDF of SINR for PC2 under Urban Macro with Fc = 29GHz

Figure 5.2.1.3-3, CDF of SINR for PC3 under Urban Macro with Fc = 29GHz

Figure 5.2.1.3-4, CDF of SINR for PC5 under Urban Macro with Fc = 29GHz

From figure 5.2.1.3-1~figure 5.2.1.3-4 we can find that 5% UE can achieve above 30dB SINR which show that PC1, PC2 and PC5 @29GHz are feasible for UL 256QAM. If the operating SNR is smaller than 30dB, then the percentage of UE meet the target SNR will increase, for example, ~10% UE can achieve above 28dB SINR even for PC3.

Observation 3: PC1, PC2, PC3 and PC5 are feasible for UL 256QAM for 29GHz if the operating SNR is not higher than 28dB.

R4-2301235

In last meeting, we provided SLS results for UL 256QAM for PC1/2/3/5 UEs of 29GHz in R4-2218595, in terms of the simulations, which are shown below:

Figure 5.2.1.3-5, CDF of SINR for PC2 under Urban Macro with Fc = 29GHz

Figure 5.2.1.3-6, CDF of SINR for PC3 under Urban Macro with Fc = 29GHz

Figure 5.2.1.3-7, CDF of SINR for PC5 under Urban Macro with Fc = 29GHz

From figure 5.2.1.3-5, 5.2.1.3-6 and 5.2.1.3-7, we can find that 5% UE can achieve above 30dB SINR which show that PC2 and PC5 @29GHz are feasible for UL 256QAM. If the operating SNR is smaller than 30dB, then the percentage of UE meet the target SNR will increase. Even ~10% UE can achieve above 28dB SINR even for PC3.

#### 5.2.1.4   Simulation results from vivo

R4-2218869

The SINR distribution for Indoor and Uma are shown in Figure 5.2.1.4-1:

Figure 5.2.1.4-1 SINR distribution

Except for PC1, The SINR of UE is hard to larger than 25 dB at the BS side.

Observation 1:

- For PC1, 20% of UE’s SINR can be larger than 25 dB.

- For PC2/PC3/PC5, <5% UE’s SINR can larger than 25 dB.

We further counted the percentage of UEs that had used 256QAM in the simulation and the throughput performance gain. The throughput gain is derived from comparing whether the 256QAM is allowed. The results are shown in below:

Figure 5.2.1.4-2 proportion and performance gain for 256 QAM

For the indoor scenario, PC1 UE can have obvious performance gain, and about 70% UE is once scheduled with 256QAM during the simulation time, but for other power classes, the throughput performance even become worse, this is because the SINR for these UE cannot support such high MCS and the BLER is bad. For the Uma scenario, even though there are many UEs scheduled with 256 QAM, the performance gain is tiny.

Observation 2:

- For the indoor scenario, only PC1 shows obvious throughput gain, and about 69.3%UEs are scheduled with 256 QAM.

- For the Uma scenario, the performance gain of all PC UEs is tiny.

R4-2301569

The simulation assumption for SLS was revised to add minimum EIRP and power control formula, and our simulation results based on such new assumptions are shown in Figure 5.2.1.4-3. The operating SNR is used as the target SNR at the BS side, and the minimum EIRP is used to calculate the conducted power level which means deducting antenna gain (including array gain).

Figure 5.2.1.4-3 SINR distribution based on SLS

For the Indoor scenario, all cases above have more than 10% UE can achieve operating SNR, but for the Uma scenario, the ratio is much lower, especially in PC2/PC5 in the 39GHz case, only about 2% UE can achieve operating SNR.

Observation 1:

- In 29GHz, 14% in Indoor and 5% in Uma PC2/PC5 UE can archive 28 dB SNR.

- In 39GHz, 12% in Indoor and 10% in Uma PC1 UE can archive 30 dB SNR; 10% in Indoor and 2% in Uma PC2/PC5 UE can archive 30 dB SNR.

#### 5.2.1.5  Simulation results from Ericsson

R4-2220036

On Figure 5.2.1.5-1 we show the results of system-level simulations for 29 GHz carrier frequency and FTP traffic model of both 10% and 30%.

Figure 5.2.1.5-1: System-level simulation results on 256QAM feasibility for PC1 UE in FR2-1 for 29 GHz carrier frequency

Note that in the simulations results, no sophisticated model was used for the misalignment between BS and UE beam directions. So, in the simulations the ideal beam management was assumed, and the impairment offset of 10 dB was added in the last stage.

By looking at the results, we can see that around 30% of UEs can achieve the operating SNR of 30 dB for FTP with 30% load and around 35% of UEs can achieve the operating SNR of 30 dB for FTP with 10% load, which means that 256 QAM for PC1 UEs in FR2-1 and 29 GHz carrier frequency is feasible.

R4-2302734

2.1  	29GHz carrier frequency

2.1.1   Indoor scenario for PC2/PC5

Observation 1: For 29GHz carrier frequency, indoor scenario and PC2/PC5, around 60% of users with FTP 10% load and 15% of users with FTP 30% load can reach the operating SNR of 28dB.

2.1.2   UMa scenario for PC2/PC5

Observation 2: For 29GHz carrier frequency, UMa scenario and PC2/PC5, around 30% of users with FTP 10% load and 28% of users with FTP 30% load can reach the operating SNR of 28dB.

2.2  	39GHz carrier frequency

2.2.1   Indoor scenario for PC2/PC5

Observation 3: For 39GHz carrier frequency, indoor scenario and PC2/PC5, around 50% of users with FTP 10% load and 12% of users with FTP 30% load can reach the operating SNR of 30dB.

2.2.2   UMa scenario for PC2/PC5

Observation 4: For 39GHz carrier frequency, UMa scenario and PC2/PC5, around 17% of users with FTP 10% load and 15% of users with FTP 30% load can reach the operating SNR of 30dB.

2.2.3   Indoor scenario for PC1

Observation 5: For 39GHz carrier frequency, indoor scenario and PC1, around 55% of users with FTP 10% load and 12% of users with FTP 30% load can reach the operating SNR of 30dB.

2.2.4   UMa scenario for PC1

Observation 6: For 39GHz carrier frequency, UMa scenario and PC1, around 50% of users with FTP 10% load and 38% of users with FTP 30% load can reach the operating SNR of 30dB.

#### 5.2.1.6  Simulation results from Xiaomi

R4-2301620

In last meeting, it is verified that the UL 256QAM for PC1 UE of 29GHz is feasible based on system simulation, but whether UL 256QAM for PC2/5 UE of 29GHz and PC1/2/5 UE of 39GHz can achieve target SNR at BS side need further confirm by system simulation. Hence, we provide the system level simulation results with the agreed target SNR of 28dB and 30dB for PC1/2/5 UE of 29GHz and 39GHz.

The UE transmit power and UL SINR CDF at 29 GHz and 39 GHz in urban macro scenario are shown in figure 5.2.1.6-1 and figure 5.2.1.6-2, respectively.

- PC1 UE with 35dBm maximum output power (n257)

- PC2/5 UE with 23dBm maximum output power  (n257)

Figure 5.2.1.6-1 Urban macro simulation results at 29 GHz

- PC1 UE with 35 dBm maximum output power (n260)

- PC2/5 UE with 23dBm maximum output power (n259)

Figure 5.2.1.6-2 Urban macro simulation results at 39 GHz

The UE transmit power and UL SINR CDF at 29 GHz and 39 GHz in indoor hotspot scenario are shown in figures 5.2.1.6-3 and 5.2.1.6-4, respectively.

- PC1 UE with 35 dBm maximum output power (n257)

- PC2/5 UE with 23dBm maximum output power (n257)

Figure 5.2.1.6-3 Indoor hotspot simulation results at 29 GHz

- PC1 UE with 35 dBm maximum output power (n260)

- PC2/5 UE with 23dBm maximum output power (n259)

Figure 5.2.1.6-4 Indoor hotspot simulation results at 39 GHz

Observation:

It can be observed from figure 5.2.1.6-1, figure 5.2.1.6-2, figure 5.2.1.6-3 and figure5.2.1.6-4 that at both 29 GHz (n257) and 39 GHz (n259 and n260):

For urban macro scenario:

-    The target SNR of 28dB for 29 GHz at BS side can be achieved by 86.95% of PC1 UE and 73.26% of PC2/5 UE.

-    The target SNR of 30dB for 39 GHz at BS side can be achieved by 82.82% of PC1 UE and 62.31% of PC2/5 UE.

-	~13% of PC1 UE and ~26% of PC2/5 UE are transmitting at maximum output power at 29 GHz (of 35 dBm and 23 dBm, respectively, for PC1 UE and PC2/5 UE).

-    ~17% of PC1 UE and ~27% of PC2/5 UE are transmitting at maximum output power at 39 GHz (of 35 dBm and 23 dBm, respectively, for PC1 UE and PC2/5 UE).

For indoor hotspot scenario:

-    The target SNR of 28dB for 29 GHz at BS side can be achieved by 100% of PC1 UE and 100% of PC2/5 UE.

-    The target SNR of 30dB for 39 GHz at BS side can be achieved by 100% of PC1 UE and 99.99% of PC2/5 UE.

-	The transmitting power of all PC1/PC2/PC5 UE are less than 20 dBm at 29 GHz.

-    The transmitting power of ~ 99% PC1/PC2/PC5 UE are less than 13 dBm at 39 GHz.

#### 5.2.1.7  Simulation results from LG Electronics

R4-2300821

-29 GHz UL 256QAM SLS

Fig.5.2.1.7-1 CDF of SINR for PC2/PC5 in urban macro scenario at 29 GHz

Fig. 5.2.1.7-2 CDF of SINR for PC2/PC5 in indoor scenario at 29 GHz

Fig. 5.2.1.7-1 and Fig. 5.2.1.7-2 show CDF of SINR for PC2/PC5 in urban macro scenario and indoor scenario at 29 GHz respectively.

Based on the SLS results, UL 256QAM at 29 GHz is feasible for both PC2 and PC5.

Observation 1: From the 29 GHz 256QAM SLS results, PC2 and PC5 can achieve SINR of 28dB in both urban macro scenario and indoor scenario.

-39 GHz UL 256QAM SLS

Fig. 5.2.1.7-3 CDF of SINR for PC1/PC2/PC5 in urban macro scenario at 39 GHz

Fig. 5.2.1.7-4 CDF of SINR for PC1/PC2/PC5 in indoor scenario at 39 GHz

Fig. 5.2.1.7-3 and Fig. 5.2.1.7-4 show CDF of SINR for PC1/PC2/PC5 in urban macro scenario and indoor scenario at 39 GHz respectively.

Based on the SLS results, UL 256QAM at 39GHz is feasible for PC1, PC2, and PC5 UEs.

Observation 2: From the 39 GHz 256QAM SLS results, PC1, PC2 and PC5 can achieve SINR of 30dB in both urban macro scenario and indoor scenario.

#### 5.2.1.8  Simulation results from Sony

R4-2302240

The SLS simulation results at 29 GHz for the indoor scenario are shown in Fig. 5.2.1.8-1. Since PC1 has been concluded to be feasible, only results of PC2/5 have been provided here.

Figure 5.2.1.8-1 The system level simulation at 29 GHz for the indoor scenario with PC2/5.

As explained, a power control scheme is adopted in the simulation, and thus the CDF plot is “capped” by the target SNR. Based on the simulation results, it can be observed that about 35% of UE can reach the target SNR, which is a significant portion of total devices. Therefore, it can be concluded that it is feasible for PC2 and PC5 to support 256 QAM at 29 GHz.

Observation 1	35% of UE can reach the target SNR condition of PC2 and PC5 at indoor scenario at 29 GHz.

Further simulations have also been performed for 39GHz for the indoor scenario, including PC1 as well, where the simulation results are shown in Fig. 5.2.1.8-2 (PC1) and Fig. 5.2.1.8-3 (PC2/5), respectively.

Figure 5.2.1.8-2 The system level simulation at 39 GHz for the indoor scenario with PC1.

Figure 5.2.1.8-3 The system level simulation at 39 GHz for the indoor scenario with PC2/5.

Based on the results shown in Fig. 5.2.1.8-2 and Fig. 5.2.1.8-3, though the target SNR and path loss have been increased, a significant portion of UE (45% for PC1 and 10% for PC2/5) can still meet the target SINR. Again, such simulation results demonstrate that a system-level gain can be obtained by supporting 256 QAM for uplink, which suggests that it is feasible to support 256 QAM in uplink is feasible for PC1/2/5 at 39 GHz.

Observation 2 	45% of PC1 UE and 10% of PC2/5 UE can meet the target SNR condition for the indoor scenario at 39 GHz.

### 5.2.2	Link level simulation

Link level simulation is targeted as mainstream way to evaluate if FR2 256QAM can achieve benefit by comparing to 64QAM.

#### 5.2.2.1    Simulation assumptions

The link level simulation assumptions are listed as in table 5.2.2.1-1, based on which, to evaluate the throughput difference between 64QAM and 256QAM. The study aims to identify conditions where UL 256QAM provides performance benefits, does not reflect side conditions for setting the UE RF requirement for UL 256QAM.

Table 5.2.2.1-1 link level simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 29 GHz (n257), 39 GHz (n260) and 48GHz (n262) |
| CBW | 50 MHz, 100MHz |
| SCS | 120 kHz |
| Allocated RBs | Full allocation |
| Propagation | TDL-A 30ns delay spread, 35Hz Doppler frequency TDL-D 30ns delay spread, 35Hz Doppler frequency Static (AWGN) |
| MCS | 64QAM: CP-OFDM: MCS 23, 24 in TS 38.214 Table 5.1.3.1-1, other MCSs are not precluded. DFT-s-OFDM: MCS 22, 23 in TS 38.214 Table 6.1.4.1-1, other MCSs are not precluded. 256QAM: CP-OFDM/DFT-s-OFDM: MCS 21, 23 in TS 38.214 Table 5.1.3.1-2, other MCSs are not precluded. Baseline: fixed MCSs |
| Symbol type | CP-OFDM; DFT-s-OFDM |
| HARQ | 8, None |
| Antenna configuration | Fading channel: 2x2 for Rank1 and Rank2, Low correlation Static channel: 1x2 for Rank1, 2x2 for Rank2 (using the diagonal matrix) |
| Channel estimation | Practical |
| Receiver type | MMSE |
| PUSCH configuration | Type A mapping, Start symbol 0, Duration 14 |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| PTRS configuration | CP-OFDM: KPTRS : 2 (every 2 RBs), LPTRS : 1 (every 1 symbol) DFT-s-OFDM: (,)=(4, 4), LPTRS : 1 (every 1 symbol) |
| Phase noise compensation | Practical based on PTRS |
| Phase noise model | TR 38.803 model (in section 6.1.10 and section 6.1.11) modelled Phase noise for TX and RX Option a): example1 (UE) + example1(BS) Option b): example2 (UE) + example2(BS) Option d): example1 (UE) + example2(BS) |
| txEVM + rxEVM excluding phase noise for 256QAM | txEVM: 3%, 3.5%, 4%, rxEVM: 3%, 3.5%, 4% Option 1: txEVM >= rxEVM; |
| Other parameters | follow assumptions in TS38.104 Section 11.2.2 . |

#### 5.2.2.2   Simulation results from Nokia

R4-2215577

The link level simulation results at 29 GHz (n257) and 39 GHz (n260) using the highlighted parameters in table 1 are shown in figures 5.2.2.2-1 and 5.2.2.2-2, respectively.

Figure 5.2.2.2-1. Simulation results at 29 GHz (n257)

Figure 5.2.2.2-2. Simulation results at 39 GHz (n260)

It can be observed from figure 5.2.2.2-1 that at 29 GHz (n257):

-	If MCS index is 21, 256QAM goes over 64QAM in around 24-25 dB for all simulated EVMs.

-	If MCS index is 23, 256QAM goes over 64QAM in around 28 dB for (transmit and receive) EVM of 3%, around 32 dB for (transmit and receive) EVM of 3.5%, and loses for (transmit and receive) EVM of 4%.

On the other hand, it can be observed from figure 5.2.2.2-2 that at 39 GHz (n260):

-	Only with (transmit and receive) EVM of 0%, MCS21 for 256QAM seems to win 64QAM in around 36 dB.

Note that the PN is challenging case at 39 GHz (n260), and due to small allocation, the number of PTRS symbols is quite low so even higher complexity ICI compensation would not really help here.

R4-2218326

The link level simulation results at 48 GHz (n262) using the highlighted parameters in table 1 with phase noise model option a and option d are shown in figures 5.2.2.2-3 and 5.2.2.2-4, respectively.

Figure 5.2.2.2-3. Simulation results with phase noise model option a

Figure 5.2.2.2-4. Simulation results with phase noise model option d

It can be observed from figures 5.2.2.2-3 and 5.2.2.2-4 that at 48 GHz (n262):

-	256QAM does not provide any throughput gain over 64QAM below 40 dB SNR.

#### 5.2.2.3   Simulation results from LG Electronics

R4-2215920

Table 5.2.2.3-1 shows the cross throughput point, where the cross throughput point indicates the SNR value at which the throughput of 64QAM and 256QAM is same, that is, after the corresponding SNR, the throughput of 256QAM is higher than 64QAM.

Table 5.2.2.3-1 Cross throughput point (29 GHz and 48 GHz, CBW:100 MHz, EVM:Tx=Rx)

Referebce: MCS24(64QAM, PUSCH Table 1, OFDM)

| Propagation MCS Mode | EVM: 2% |  | EVM: 3% |  | EVM: 3.5% |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 29 GHz | 48 GHz | 29 GHz | 48 GHz | 29 GHz | 48 GHz |
| TDL-A MCS 21 (256 QAM, PDSCH Table 2, OFDM) | 24.5 dB | N/A | 27 dB | N/A | 30 dB | N/A |
| TDL-A MCS 23 (256 QAM, PDSCH Table 2, OFDM) | 29 dB | N/A | N/A | N/A | N/A | N/A |
| TDL-D MCS 21 (256 QAM, PDSCH Table 2, OFDM) | 18.5 dB | 30 dB | 20 dB | N/A | 21 dB | N/A |
| TDL-D MCS 23 (256 QAM, PDSCH Table 2, OFDM) | 23 dB | N/A | 24.5 dB | N/A | 27 dB | N/A |
| AWGN MCS 21 (256 QAM, PDSCH Table 2, OFDM) | 16 dB | 22.5 dB | 16.5 dB | 24 dB | 17 dB | 27.5dB |
| AWGN MCS 23 (256 QAM, PDSCH Table 2, OFDM) | 19 dB | N/A | 20 dB | N/A | 21 dB | N/A |

29 GHz (n257), EVM(Tx=Rx):3.5%

| Fig 5.2.2.3-1. Throughput performance between 256QAM and 64QAM in TDL-A | Fig 5.2.2.3-2. Throughput performance between 256QAM and 64QAM in TDL-D |
| --- | --- |
| Fig 5.2.2.3-3. Throughput performance between 256QAM and 64QAM in AWGN |  |

29 GHz (n257), EVM(Tx=Rx):3%

| Fig 5.2.2.3-4. Throughput performance between 256QAM and 64QAM in TDL-A | Fig 5.2.2.3-5. Throughput performance between 256QAM and 64QAM in TDL-D |
| --- | --- |
| Fig 5.2.2.3-6. Throughput performance between 256QAM and 64QAM in AWGN |  |

29 GHz (n257), EVM(Tx=Rx):2%

| Fig 5.2.2.3-7. Throughput performance between 256QAM and 64QAM in TDL-A | Fig 5.2.2.3-8. Throughput performance between 256QAM and 64QAM in TDL-D |
| --- | --- |
| Fig 5.2.2.3-9. Throughput performance between 256QAM and 64QAM in AWGN |  |

48 GHz (n262), EVM(Tx=Rx):3.5%

| Fig 5.2.2.3-10. Throughput performance between 256QAM and 64QAM in TDL-A | Fig 5.2.2.3-11. Throughput performance between 256QAM and 64QAM in TDL-D |
| --- | --- |
| Fig 5.2.2.3-12. Throughput performance between 256QAM and 64QAM in AWGN |  |

48 GHz (n262), EVM(Tx=Rx):3%

| Fig 5.2.2.3-13. Throughput performance between 256QAM and 64QAM in TDL-A | Fig 5.2.2.3-14. Throughput performance between 256QAM and 64QAM in TDL-D |
| --- | --- |
| Fig 5.2.2.3-15. Throughput performance between 256QAM and 64QAM in AWGN |  |

48 GHz (n262), EVM(Tx=Rx):2%

| Fig 5.2.2.3-16. Throughput performance between 256QAM and 64QAM in TDL-A | Fig 5.2.2.3-17. Throughput performance between 256QAM and 64QAM in TDL-D |
| --- | --- |
| Fig 5.2.2.3-18. Throughput performance between 256QAM and 64QAM in AWGN |  |

From the simulation results, the followings are observed for UL 256QAM performance gain,

Observation1: In 29 GHz & TDL-A,

- There is the performance gain in MCS21 when EVM ≥ 3.0%

- There is no performance gain in MCS23

Observation2: In 29 GHz & TDL-D

- There is the perfornace gain when EVM = 3.5%

Observation3: In 29 GHz & AWGN

- There is the perfornace gain at a relatively low SNR when EVM = 3.5%.

Observation4: In 48 GHz,

- There is no performance gain in most cases

R4-2218681

Table 5.2.2.3-2 Cross throughput point (29 GHz and 39 GHz, CBW:100 MHz, EVM:Tx=Rx=3.5%)

Referebce: MCS24(64QAM, PUSCH Table 1, OFDM)

| Propagation Mode | 29 GHz |  |  |  | 39 GHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MCS 20 | MCS 21 | MCS 22 | MCS 23 | MCS 20 | MCS 21 | MCS 22 | MCS 23 |
| TDL-A (256 QAM, PDSCH Table 2, OFDM) (dB) | 25.5 | 30 | N/A | N/A | N/A | N/A | N/A | N/A |
| TDL-D (256 QAM, PDSCH Table 2, OFDM) (dB) | 18.5 | 21 | 23 | 27 | 21 | 24 | 30 | N/A |

29 GHz (n257)

Fig5.2.2.3-19. Throughput performance between 256QAM and 64QAM in TDL-A

Fig5.2.2.3-20. Throughput performance between 256QAM and 64QAM in TDL-D

39 GHz (n260)

Fig5.2.2.3-21. Throughput performance between 256QAM and 64QAM in TDL-A

Fig5.2.2.3-22. Throughput performance between 256QAM and 64QAM in TDL-D

Observation 2: In 29GHz TDL-A, the throughput gain of 256QAM is shown in MCS index 20 and 21, and operating SNR can vary from 25.5dB to 30dB depending on the MCS index number.

Observation 3: In 29GHz TDL-D, the throughput gain of 256QAM is shown in MCS index 20~23, and operating SNR can vary from 18.5dB to 27dB depending on the MCS index number.

Observation 4: In 39GHz TDL-A, no throughput gain of 256QAM  is shown in MCS index 20~23.

Observation 5: In 39GHz TDL-D, the throughput gain of 256QAM is shown in MCS index 20 ~22, and operating SNR can vary from 21dB to 30dB depending on the MCS index number.

Observation 6: If the operating SNR is considered based on TDL-A, SNR of 25.5 dB or more need to be set for operation at 29GHz, however it is not practical. And N/A is shown at 39GHz, so it is also not practical.

Observation 7: If the operating SNR is considered based on TDL-D with limited MCS, an operating SNR value of 18.5dB to 27dB at 29GHz may be considered, and an operating SNR value of 21dB to 30dB at 39GHz may be considered.

#### 5.2.2.4   Simulation results from vivo

R4-2216128

- 29 GHz

- TDL-A

- TDL-D

- AWGN

The simulation results above are summarized as follows.

Observation 1：The UL 256 QAM under 29 GHz can achieve performance gain at:

| Tx_EVM = Rx_EVM =3% | TDL-A |  | TDL-D |  | AWGN |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100MHz | 50 MHz | 100MHz | 50 MHz | 100MHz |
| DFT-s-OFDM | 22.0 dB | 22.8 dB | 20 dB | 21 dB | 18 dB | 19.2 dB |
| CP-OFDM | 22.5 dB | 23.8 dB | 20.3 dB | 21.5 dB | 19.2 dB | 19.7 dB |

| Tx_EVM = Rx_EVM =3.5% | TDL-A |  | TDL-D |  | AWGN |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100MHz | 50 MHz | 100MHz | 50 MHz | 100MHz |
| DFT-s-OFDM | 22.2 dB | 23.1 dB | 20.3 dB | 21.2 dB | 18.5 dB | 19.5 dB |
| CP-OFDM | 22.7 dB | 24 dB | 20.8 dB | 21.8 dB | 19.5 dB | 19.7 dB |

| Tx_EVM = Rx_EVM =4% | TDL-A |  | TDL-D |  | AWGN |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100MHz | 50 MHz | 100MHz | 50 MHz | 100MHz |
| DFT-s-OFDM | 22.9 dB | 23.5 dB | 20.6 dB | 21.5 dB | 19.2 dB | 19.7 dB |
| CP-OFDM | 23.2 dB | 24.7 dB | 21.2 dB | 22 dB | 19.8 dB | 19.9 dB |

- 39 GHz

- TDL-A

- TDL-D

- AWGN

Observation 2：The UL 256 QAM under 39 GHz can achieve performance gain at:

| Tx_EVM = Rx_EVM =3% | TDL-A |  | TDL-D |  | AWGN |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100MHz | 50 MHz | 100MHz | 50 MHz | 100MHz |
| DFT-s-OFDM | 24 dB | 27 dB | 22.3 dB | 25.8 dB | 19.9 dB | 22 dB |
| CP-OFDM | 28 dB | N/A | 24 dB | N.A. | 21.5 dB | 26 dB |

| Tx_EVM = Rx_EVM =3.5% | TDL-A |  | TDL-D |  | AWGN |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100MHz | 50 MHz | 100MHz | 50 MHz | 100MHz |
| DFT-s-OFDM | 24.4 dB | 27.5 dB | 23 dB | 26 dB | 20 dB | 23 dB |
| CP-OFDM | 29.2 dB | N/A | 25 dB | N.A. | 21.8 dB | 28 dB |

| Tx_EVM = Rx_EVM =4% | TDL-A |  | TDL-D |  | AWGN |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100MHz | 50 MHz | 100MHz | 50 MHz | 100MHz |
| DFT-s-OFDM | 24.8 dB | 28.2 dB | 23.3 dB | 28 dB | 20.3 dB | 23 dB |
| CP-OFDM | 34 dB | N/A | 25.8 dB | N.A. | 22 dB | 30 dB |

- 48 GHz

- TDL-A

- TDL-D

- AWGN

Observation 3：The UL 256 QAM is hard to bring performance gain under 48 GHz

#### 5.2.2.5   Simulation results from Huawei

R4-2216245

The link lever simulation results are shown in Figure5.2.2.5-1, Figure 5.2.2.5-2 and Figure 5.2.2.5-3 for TDL-D in different frequency ranges. From the simulation results, it is shown that support 256 QAM can provide significant performance gain over 64QAM where the UE is in good propagation condition.

Figure 5.2.2.5-1 Throughput comparison of 256QAM and 64QAM for RANK1_29 GHz

Figure 5.2.2.5-2 Throughput comparison of 256QAM and 64QAM for RANK1_39GHz

Figure 5.2.2.5-3 Throughput comparison of 256QAM and 64QAM for RANK1_48GHz

R4-2219144

The link lever simulation results are shown in Figure 5.2.2.5-4, Figure 5.2.2.5-5, Figure 5.2.2.5-6 and Figure 5.2.2.5-7 for TDL-D in different frequency ranges. From the simulation results, it is shown that support 256 QAM can provide significant performance gain over 64QAM where the UE is in good propagation condition.

Figure 5.2.2.5-4 Throughput comparison of MCS 21 256QAM and 64QAM for RANK1_29 GHz

Figure5.2.2.5-5 Throughput comparison of MCS 23 256QAM and 64QAM for RANK1_29 GHz

Figure 5.2.2.5-6 Throughput comparison of MCS 21 256QAM and 64QAM for RANK1_39GHz

Figure 5.2.2.5-7 Throughput comparison of MCS 23 256QAM and 64QAM for RANK1_39GHz

#### 5.2.2.6   Simulation results from Sony [R4-2216251]

R4-2216251

We performed simulations and compared the throughput between 64QAM and 256QAM for three different transmit and receive EVM values (3%, 3.5%, and 4%). TX EVM has been the same as RX EVM in each simulation. The simulations are done according to the agreed simulation assumptions [3]. Only Rank-1, and AWGN for 29 GHz, respectively 48 GHz, are simulated so far. 100MHz channel BW and no HARQ have been used. Phase noise is added according to option 2 in the simulation assumptions, i.e., example2, at both the UE and BS.

- 29 GHz

We performed simulations to address whether 256-QAM works in FR2-1 and to demonstrate its performance gain. The result is shown in Figure 5.2.2.6-1.

Figure 5.2.2.6-1 Throughput comparing 64QAM and 256QAM with AWGN for 29GHz, EVM 3%, 3.5% and 4%.

Figure 5.2.2.6-1 shows that throughput for 64QAM (red curve) has already saturated at an SNR level of 16dB. For 256QAM, SNR levels of 22 dB (EVM=3%), 23 dB (3.5%) and 24 dB (EVM=4%) are required to exceed the throughput performance of 64QAM. The simulations have been carried out with a simple receiver, and further studies of advanced receivers are needed to determine whether the above results could be improved.

Observation 1	For 256QAM to exceed throuput performance of 64QAM SNR levels of 22 dB (EVM=3%) to 24 dB (EVM=4%) are required.

R4-2218753

We performed simulations to determine whether 256-QAM works for FR2-1 and demonstrate its performance gain. Figure 5.2.2.6-2 shows the results.

Figure 5.2.2.6-2 Throughput comparing 64QAM and 256QAM with AWGN for 29GHz, EVM 3.5%.

Figure 5.2.2.6-2 shows that throughput for 64QAM (red curves) has already saturated at an SNR level of 16dB. For 256QAM, SNR levels of 20 dB for MCS21 and 25 dB for MCS23 (EVM 3.5%) are required to exceed the throughput performance of 64QAM. To reach throughput saturation for 256QAM, SNR levels of 21dB for MCS21 and 28 dB for MCS23 are needed.

Observation 1	For 256QAM, to reach saturation in throughput performance, SNR levels of 21dB for MCS21 and 28dB for MCS23 are needed for 29GHz.

Simulations have also been performed for 39GHz with the same setup as for 29GHz. Figure 5.2.2.6-3 shows the results.

Figure 5.2.2.6-3	 Throughput comparing 64QAM and 256QAM with AWGN for 39GHz, EVM 3.5%.

Figure 5.2.2.6-3 shows 256QAM performance for 39GHz compared to 64QAM. SNR levels of 21.5dB are needed to exceed the throughput performance of 64QAM for MCS21. For MCS23, an SNR of 36dB is required to exceed the throughput performance of 64QAM. An SNR level of 24dB is needed to reach throughput saturation for 256QAM for MCS21. For MCS23, we didn’t see saturation within the simulated SNR range.

Observation 2	For 256QAM, to reach saturation in throughput performance, SNR levels of 24dB for MCS21 are needed for 39GHz. To exceed the performance of 64QAM, an SNR of 21.5dBm is required.

#### 5.2.2.7   Simulation results from Xiaomi

R4-2216350

We compare the UL 256QAM and UL 64QAM performance under different channel conditions and carrier frequencies to determine whether UL 256QAM can provide gains over UL 64QAM for FR2-1. For all simulation, we assumed 50MHz channel bandwidth with 120kHz SCS and phase noise model of example2 (UE) + example2(BS) based on above simulation assumptions.

- CP-OFDM

- 29GHz

Figure 5.2.2.7-1 Throughput performance for 29GHz under AWGN, CP-OFDM

Figure 5.2.2.7-2 Throughput performance for 29GHz under TDL-D, CP-OFDM

Figure 5.2.2.7-3 Throughput performance for 29GHz under TDL-A, CP-OFDM

- 39GHz

Figure 5.2.2.7-4 Throughput performance for 39GHz under AWGN, CP-OFDM

Figure 5.2.2.7-5 Throughput performance for 39GHz under TDL-D, CP-OFDM

Figure 5.2.2.7-6 Throughput performance for 39GHz under TDL-A, CP-OFDM

- 48GHz

Figure 5.2.2.7-7 Throughput performance for 48GHz under AWGN, CP-OFDM

- DFT-s-OFDM

- 29GHz

Figure 5.2.2.7-8 Throughput performance for 29GHz under AWGN, DFT-s-OFDM

Figure 5.2.2.7-9 Throughput performance for 29GHz under TDL-D, DFT-s-OFDM

Figure 5.2.2.7-10 Throughput performance for 29GHz under TDL-A, DFT-s-OFDM

- 39GHz

Figure 5.2.2.7-11 Throughput performance for 39GHz under AWGN, DFT-s-OFDM

Figure 5.2.2.7-12 Throughput performance for 39GHz under TDL-D, DFT-s-OFDM

Figure 5.2.2.7-13 Throughput performance for 39GHz under TDL-A, DFT-s-OFDM

- 48GHz

Figure 5.2.2.7-14 Throughput performance for 39GHz under TDL-A, DFT-s-OFDM

From the simulation results, table 5.2.2.7-1 summary the SNR required to achieve gains for 256QAM compare to 64QAM:

Table 5.2.2.7-1 UL 256QAM required SNR to achieve gains compare to 64QAM

| Parameters for UL 256QAM |  |  | SNR required to achieve gains for 256QAM [dB] |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 29GHz |  |  |  |  | 39GHz |  |  | 48GHz |  |  |
| Channel | MCS |  |  | 3%+3% | 3.5%+3.5% | 4%+4% | 3%+3% |  | 3.5%+3.5% | 4%+4% | 3%+3% | 3.5%+3.5% | 4%+4% |  |
| AWGN 1x2 RANK1 | MCS21 | CP-OFDM |  | 19.3 | 19.5 | 19.6 | 20 |  | 21.5 | 22 | 24 | 25 | 28 |  |
|  |  | DFT-OFDM |  | 19.3 | 19.5 | 19.6 | 20.1 |  | 21.1 | 21.7 | 22.5 | 23.6 | 25 |  |
|  | MCS23 | CP-OFDM |  | 23 | 23.5 | 25.5 | NA |  | NA | NA | NA | NA | NA |  |
|  |  | DFT-OFDM |  | 22.5 | 23.5 | 24.7 | NA |  | NA | NA | NA | NA | NA |  |
| TDL-D 2x2 RANK2 | MCS21 | CP-OFDM |  | 21.8 | 22.5 | 23.1 | 24.1 |  | 25.2 | 28 | - | - | - |  |
|  |  | DFT-OFDM |  | 22 | 22.6 | 23.3 | 24.1 |  | 25.7 | 28 | - | - | - |  |
|  | MCS23 | CP-OFDM |  | 25.8 | 27.3 | NA | NA |  | NA | NA |  |  |  |  |
|  |  | DFT-OFDM |  | 25.7 | 26.1 | 28 | NA |  | NA | NA | - | - | - |  |
| TDL-A 2x2 RANK2 | MCS21 | CP-OFDM |  | 21.8 | 22.7 | 23.8 | 25.2 |  | 26.7 | NA | - | - | - |  |
|  |  | DFT-OFDM |  | 21.8 | 22.7 | 23.8 | NA |  | NA | NA | - | - | - |  |
|  | MCS23 | CP-OFDM |  | 26.5 | 28 | NA | NA |  | NA | NA | - | - | - |  |
|  |  | DFT-OFDM |  | 25.8 | 27.5 | NA | NA |  | NA | NA | - | - | - |  |
| - means the related simulations haven’t been done due to time limit. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Observation:

And it is shown that supporting UL 256 QAM can provide significant performance gain over UL 64QAM for 3.5% EVM.

For AWGN channel with 3.5% Tx EVM+3.5% Rx EVM, a SNR of >19.5 dB is needed for 29GHz, a SNR of >21.1 dB is needed for 39GHz and a SNR of >23.6 dB is needed for 48GHz.

For TDL-A and TDL-D fading channel with 3.5% Tx EVM+3.5% Rx EVM, a SNR of >22.5 dB is needed for 29GHz, a SNR of >25.2 dB is needed for 39GHz, due to lime limit, the related simulation for 48GHz haven’t been done.

R4-2219075

We further compare the UL 256QAM and UL 64QAM performance under different channel conditions and carrier frequencies to determine whether UL 256QAM can provide gains over UL 64QAM for 48GHz. For all simulations, we assumed 50MHz channel bandwidth with 120kHz SCS and phase noise model of example2 (UE) + example2(BS) based on the agreed simulation assumptions [2].

- CP-OFDM

Figure 5.2.2.7-15 Throughput performance for 48GHz under AWGN, CP-OFDM

Figure 5.2.2.7-16 Throughput performance for 48GHz under TDL-D, CP-OFDM

Figure 5.2.2.7-17 Throughput performance for 48GHz under TDL-D, CP-OFDM

- DFT-s-OFDM

Figure 5.2.2.7-18 Throughput performance for 48GHz under AWGN, DFT-s-OFDM

Figure 5.2.2.7-19 Throughput performance for 48GHz under TDL-D, DFT-s-OFDM

Figure 5.2.2.7-20 Throughput performance for 48GHz under TDL-A, DFT-s-OFDM

From the simulation results for 48GHz, table 5.2.2.7-2 updated the SNR required to achieve gains for 256QAM compare to 64QAM:

Table 5.2.2.7-2 UL 256QAM required SNR to achieve gains compare to 64QAM

| Parameters for UL 256QAM |  |  | 48GHz SNR required to achieve gains for 256QAM [dB] |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel | MCS |  | 3%+3% | 3.5%+3.5% | 4%+4% |
| AWGN 1x2 RANK1 | MCS21 | CP-OFDM | 24 | 25 | 28 |
|  |  | DFT-OFDM | 22.5 | 23.6 | 25 |
|  | MCS23 | CP-OFDM | NA | NA | NA |
|  |  | DFT-OFDM | NA | NA | NA |
| TDL-D 2x2 RANK2 | MCS21 | CP-OFDM | NA | NA | NA |
|  |  | DFT-OFDM | NA | NA | NA |
|  | MCS23 | CP-OFDM | NA | NA | NA |
|  |  | DFT-OFDM | NA | NA | NA |
| TDL-A 2x2 RANK2 | MCS21 | CP-OFDM | NA | NA | NA |
|  |  | DFT-OFDM | NA | NA | NA |
|  | MCS23 | CP-OFDM | NA | NA | NA |
|  |  | DFT-OFDM | NA | NA | NA |

#### 5.2.2.8   Simulation results from ZTE

R4-2216426

To study the gain and operating SNR, link level simulation approach was adopted to compare 256QAM with 64QAM. The simulation assumptions are from 5.2.2.1, and the parameters used in this simulation are marked in red. The throughput curves for CP-OFDM Rank1 transmission are listed in figure 5.2.2.8-1~ figure  5.2.2.8-9 for reference.

Figure  5.2.2.8-1: Throughput performance comparison between FR2 UL 256QAM and FR2 UL 64QAM @29GHz AWGN channel

Figure 5.2.2.8-2: Throughput performance comparison between FR2 UL 256QAM and FR2 UL 64QAM @29GHz TDLD channel

Figure 5.2.2.8-3: Throughput performance comparison between FR2 UL 256QAM and FR2 UL 64QAM @29GHz TDLA channel

Figure 5.2.2.8-4: Throughput performance comparison between FR2 UL 256QAM and FR2 UL 64QAM @39GHz AWGN channel

Figure 5.2.2.8-5: Throughput performance comparison between FR2 UL 256QAM and FR2 UL 64QAM @39GHz TDLD channel

Figure 5.2.2.8-6: Throughput performance comparison between FR2 UL 256QAM and FR2 UL 64QAM @39GHz TDLA channel

Figure 5.2.2.8-7: Throughput performance comparison between FR2 UL 256QAM and FR2 UL 64QAM @48GHz AWGN channel

Figure 5.2.2.8-8: Throughput performance comparison between FR2 UL 256QAM and FR2 UL 64QAM @48GHz TDLD channel

Figure 5.2.2.8-9: Throughput performance comparison between FR2 UL 256QAM and FR2 UL 64QAM @48GHz TDLA channel

The target SNR values are summarized in table 5.2.2.8-1, where “-” means that 256QAM can not provide better performance in the given SNR range compared to 64QAM.

Table 5.2.2.8-1. Target SNR for CP-OFDM, Rank 1 transmission

| index | frequency | Channel | Test setup (64QAM/256QAM) | Target SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | EVM 3.0+3.0 | EVM 3.5+3.5 | EVM 4.0+4.0 |
| 1 | 29GHz | AWGN | MCS23/21 | 21.31 | 21.71 | 22.19 |
| 2 |  |  | MCS24/23 | 25.92 | 27.66 | 31.06 |
| 3 |  | TDL-D | MCS23/21 | 23.42 | 23.73 | 24.16 |
| 4 |  |  | MCS24/23 | 28.44 | 30.10 | 34.28 |
| 5 |  | TDL-A | MCS23/21 | 25.90 | 26.45 | 27.23 |
| 6 |  |  | MCS24/23 | 31.49 | 34.31 | - |
| 7 | 39GHz | AWGN | MCS23/21 | 21.28 | 21.64 | 21.95 |
| 8 |  |  | MCS24/23 | 25.86 | 27.64 | 30.62 |
| 9 |  | TDL-D | MCS23/21 | 26.36 | 27.50 | 29.23 |
| 10 |  |  | MCS24/23 | - | - | - |
| 11 |  | TDL-A | MCS23/21 | 30.48 | 32.35 | 36.62 |
| 12 |  |  | MCS24/23 | - | - | - |
| 13 | 48GHz | AWGN | MCS23/21 | - | - | - |
| 14 |  |  | MCS24/23 | - | - | - |
| 15 |  | TDL-D | MCS23/21 | - | - | - |
| 16 |  |  | MCS24/23 | - | - | - |
| 17 |  | TDL-A | MCS23/21 | - | - | - |
| 18 |  |  | MCS24/23 | - | - | - |

Based on the simulation results, the following observations are given:

Observation 1: For 29GHz:

256QAM performance gain can be expected in the following cases:

- AWGN and TDL-D channel,

- TDL-A channel when MCS21(256QAM)/MCS23(64QAM) are selected,

- TDL-A channel when MCS23(256QAM)/MCS24(64QAM) and EVM3.0+3.0 or EVM3.5+3.5 are selected

However, 256QAM performance gain can not be expected in the following cases:

- TDL-A channel when MCS23(256QAM)/MCS24(64QAM) and EVM4.0+4.0 are selected.

Observation 2: For 39GHz:

256QAM performance gain can be expected in the following cases:

- AWGN

- TDL-D and TDL-A channel when MCS21(256QAM)/MCS23(64QAM) are selected

However, 256QAM performance gain can not be expected in the following cases:

- TDL-D and TDL-A channel when MCS23(256QAM)/MCS24(64QAM) are selected.

For 48GHz, using the same phase noise model, the phase noise becomes larger as frequency increases.

Observation 3: For 48GHz:

256QAM performance gain can not be expected for AWGN, TDL-D and TDL-A channel for all the MCS.

R4-2218595

For MCS22 evaluation, the simulation assumptions are 5.2.2.1, and the parameters used in this simulation are marked in red. The throughput curves for MCS22 for CP-OFDM Rank1 transmission are listed in figure5.2.2.8-10~figure 5.2.2.8-12.

Figure 5.2.2.8-10, Throughput performance comparison between FR2 UL 256QAM MCS 22 and FR2 UL 64QAM MCS24 @39GHz AWGN channel

Figure 5.2.2.8-12, Throughput performance comparison between FR2 UL 256QAM MCS 22 and FR2 UL 64QAM MCS24 @39GHz TDL-D channel

Figure 5.2.2.8-12, Throughput performance comparison between FR2 UL 256QAM MCS 22 and FR2 UL 64QAM MCS24 @39GHz TDL-A channel

The updated target SNR values are summarized in table 5.2.2.8-2, where “-” means that 256QAM can not provide better performance in the given SNR range compared to 64QAM. As EVM was reached in last meeting so the simulation results of EVM 3.0% and EVM 4.0% are not provided.

Table 5.2.2.8-2. Target SNR for CP-OFDM, Rank 1 transmission

| index | frequency | Channel | Test setup (64QAM/256QAM) | Target SNR |
| --- | --- | --- | --- | --- |
|  |  |  |  | EVM 3.5%+3.5% |
| 1 | 29GHz | AWGN | MCS23/21 | 20.80 |
| 2 |  |  | MCS24/23 | 27.47 |
| 3 |  | TDL-D | MCS23/21 | 22.42 |
| 4 |  |  | MCS24/23 | 27.71 |
| 5 |  | TDL-A | MCS23/21 | 24.03 |
| 6 |  |  | MCS24/23 | 29.07 |
| 7 | 39GHz | AWGN | MCS23/21 | 22.00 |
|  |  |  | MCS24/22 | 27.55 |
| 8 |  |  | MCS24/23 | - |
| 9 |  | TDL-D | MCS23/21 | 26.00 |
|  |  |  | MCS24/22 | 29.61 |
| 10 |  |  | MCS24/23 | - |
| 11 |  | TDL-A | MCS23/21 | 26.09 |
|  |  |  | MCS24/22 | 30.70 |
| 12 |  |  | MCS24/23 | - |
| 13 | 48GHz | AWGN | MCS23/21 | - |
| 14 |  |  | MCS24/23 | - |
| 15 |  | TDL-D | MCS23/21 | - |
| 16 |  |  | MCS24/23 | - |
| 17 |  | TDL-A | MCS23/21 | - |
| 18 |  |  | MCS24/23 | - |

Based on the simulation results, the following observations are given:

Observation 1: For 39GHz:

UL 256QAM performance gain can be expected in the following cases:

- AWGN, TDL-D and TDL-A channel when MCS21(256QAM)/MCS23(64QAM) are selected

- AWGN, TDL-D and TDL-A channel when MCS22(256QAM)/MCS24(64QAM) are selected

Observation 2: UL 256QAM performance gain can not be expected for 48GHz.

#### 5.2.2.9   Simulation results from Ericsson

R4-2220036

On Figure 5.2.2.9-1 – Figure 5.2.2.9-8 we provide simulation results for various scenarios described in figure captions and figure legends.

Figure 5.2.2.9-1: 29GHz, 100MHz, No HARQ, AWGN 2x2 Rank 1 channel, txEVM= rxEVM=3.5%

Figure 5.2.2.9-2: 29GHz, 100MHz, No HARQ, Fading 2x2 Rank 1 channel, txEVM= rxEVM=3.5%

Figure 5.2.2.9-3: 29GHz, 100MHz, No HARQ, AWGN 2x2 Rank 2 channel, txEVM= rxEVM=3.5%

Figure 5.2.2.9-4: 29GHz, 100MHz, No HARQ, Fading 2x2 Rank 2 channel, txEVM= rxEVM=3.5%

Figure 5.2.2.9-5: 29GHz, 100MHz, HARQ=8, AWGN 2x2 Rank 1 channel, txEVM= rxEVM=3.5%

Figure 5.2.2.9-6: 29GHz, 100MHz,  HARQ=8, Fading 2x2 Rank 1 channel, txEVM= rxEVM=3.5%

Figure 5.2.2.9-7: 29GHz, 100MHz, HARQ=8, AWGN 2x2 Rank 2 channel, txEVM= rxEVM=3.5%

Figure 5.2.2.9-8: 29GHz, 100MHz, HARQ=8, Fading 2x2 Rank 2 channel, txEVM= rxEVM=3.5%

In Table 5.2.2.9-1 we have summarized results where we indicate two operating SNR points where 256QAM MCS21 outperforms 64QAM MCS23 and 64QAM MCS24, respectively.

| HARQ Type | Channel type | Cross-point between 256QAM MCS21 and 64QAM MCS23 | Cross-point between 256QAM MCS21 and 64QAM MCS24 |
| --- | --- | --- | --- |
| No HARQ | AWGN 2x2 Rank 1 | 26.2 dB | 26.8 dB |
|  | AWGN 2x2 Rank 2 | 28.7 dB | 29 dB |
|  | Fading 2x2 Rank 1 | 27.9 dB | 28.1 dB |
|  | Fading 2x2 Rank 2 | 32.4 dB | 33.2 dB |
| HARQ=8 | AWGN 2x2 Rank 1 | 26 dB | 26.4 dB |
|  | AWGN 2x2 Rank 2 | 28.1 dB | 28.8 dB |
|  | Fading 2x2 Rank 1 | 27.1 dB | 27.8 dB |
|  | Fading 2x2 Rank 2 | 31.2 dB | 32.2 dB |

Table 5.2.2.9-1: Summary of link-level simulation results

We conclude first that between not having HARQ and having HARQ=8 repetitions there are no big differences when it comes to the cross-point (operating SNR) between 256QAM MCS21 and 64QAM MCS23/MCS24, respectively. For the AWGN channel, the operating SNR is lower by 0.3 dB in average in the “No HARQ” case compared with “HARQ=8” case, while for the TDL-D fading channel the operating SNR is lower by 0.8 dB in the “No HARQ” case compared with “HARQ=8” case.

For “No HARQ” case, for AWGN channel the average operating SNR is equal to 27.7 dB.

For “No HARQ” case, for TDL-D fading channel the average operating SNR is equal to 30.4 dB.

#### 5.2.2.10   Summary for 29GHz simulation results

| Parameter |  |  | Nokia R4-2215577 | LG Electronics R4-2215920 R4-2218681 | vivo R4-2216128 | Huawei R4-2216245 R4-2219144 | Sony R4-2216251 R4-2218753 | Xiaomi R4-2216350 | ZTE R4-2216426 R4-2218595 | Ericsson R4-2220036 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency |  |  | 29 GHz | 29 GHz | 29 GHz | 29 GHz | 29 GHz | 29 GHz | 29 GHz | 29 GHz |
| CBW |  |  | 50MHz | 100MHz | 50MHz/100MHz | 50MHz | 100MHz | 50MHz | 100MHz | 100MHz |
| SCS |  |  | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz |
| HARQ |  |  | none | none |  | none | none | none | none | none/8 |
| RANK |  |  | RANK1 | RANK1 | RANK1 | RANK1 | RANK1 | RANK1 | RANK1 | RANK1 |
| Phase noise model |  |  | Option a): example1 (UE) + example1(BS) | Option b): example2 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option d): example1 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option a): example1 (UE) + example1(BS) |
| Tx EVM=Rx EVM |  |  | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% |
| Target SNR(dB) | TDL-A | MCS20 |  | 25.5 |  |  |  |  |  |  |
|  |  | MCS21 |  | 30 | 22.7/24 |  |  | 22.7 | 26.45/24.03 |  |
|  |  | MCS22 |  | NA |  |  |  |  |  |  |
|  |  | MCS23 |  | NA | 26.3/28.4 |  |  | 28 | 34.31/29.07 |  |
|  | TDL-D | MCS20 |  | 18.5 |  |  |  |  |  |  |
|  |  | MCS21 | 24-25 | 21 | 20.8/21.8 | 20.1 | 23 | 22.5 | 23.73/22.42 | 27.9/27.1 |
|  |  | MCS22 |  | 23 |  |  |  |  |  |  |
|  |  | MCS23 | 32 | 27 | 25.7/28 | 23 |  | 27.3 | 30.10/27.71 |  |
|  | AWGN | MCS21 |  | 17 | 19.5/19.7 |  | 20 | 19.5 | 21.71/20.8 | 26.2/26 |
|  |  | MCS23 |  | 21 | 22.2/23.8 |  | 25 | 23.5 | 27.66/27.47 |  |

Average value for 29GHz operating SNR based on TDL-D:

|  | Operating SNR (dB) with EVM 3.5% |  |  |  |
| --- | --- | --- | --- | --- |
|  | TDL-D, MCS20 | TDL-D, MCS21 | TDL-D, MCS22 | TDL-D, MCS23 |
| Nokia | - | 25 | - | 32 |
| LGE | 18.5 | 21 | 23 | 27 |
| vivo | - | 21.8 | - | 28 |
| Huawei | - | 20.1 | - | 23 |
| Xiaomi | - | 22.5 | - | 27.3 |
| ZTE | - | 22.42 | - | 27.71 |
| Ericsson | - | 27.9 | - | - |
| Average | 18.5 | 23 | 23 | 27.5 |

#### 5.2.2.11   Summary for 39GHz simulation results

| Parameter |  |  | Nokia R4-2215577 | LG Electronics R4-2218681 | vivo R4-2216128 | Huawei R4-2216245 R4-2219144 | Sony R4-2218753 | Xiaomi R4-2216350 | ZTE R4-2216426 R4-2218595 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency |  |  | 39 GHz | 39 GHz | 39 GHz | 39 GHz | 39 GHz | 39 GHz | 39 GHz |
| CBW |  |  | 50MHz | 100MHz | 50MHz/100MHz | 50MHz | 50MHz | 50MHz | 100MHz |
| SCS |  |  | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz |
| HARQ |  |  | none | none | none | none | none | none | none |
| RANK |  |  | RANK1 | RANK1 | RANK1 | RANK1 | RANK1 | RANK1 | RANK1 |
| Phase noise model |  |  | Option a): example1 (UE) + example1(BS) | Option b): example2 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option d): example1 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) |
| Tx EVM=Rx EVM |  |  | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% |
| Target SNR(dB) | TDL-A | MCS20 |  | NA |  |  |  |  |  |
|  |  | MCS21 |  | NA | 29.2/NA |  |  | 26.7 | 32.35/26.09 |
|  |  | MCS22 |  | NA |  |  |  |  | 30.70 |
|  |  | MCS23 |  | NA | NA |  |  | NA | NA |
|  | TDL-D | MCS20 |  | 21 |  |  |  |  |  |
|  |  | MCS21 | 36 | 24 | 25/NA | 21 |  | 25.2 | 27.50/26 |
|  |  | MCS22 |  | 30 |  |  |  |  | 29.61 |
|  |  | MCS23 |  | NA | NA | 25.4 |  | NA | NA |
|  | AWGN | MCS21 |  |  | 21.8/28 |  | 21.5 | 21.5 | 21.64/22 |
|  |  | MCS22 |  |  |  |  |  |  | 27.55 |
|  |  | MCS23 |  |  | NA |  | 36 | NA | 27.64 |

Average value for 39GHz operating SNR based on TDL-D:

|  | Operating SNR (dB) with EVM 3.5% |  |  |  |
| --- | --- | --- | --- | --- |
|  | TDL-D, MCS20 | TDL-D, MCS21 | TDL-D, MCS22 | TDL-D, MCS23 |
| Nokia | - | 36 | - | - |
| LGE | 21 | 24 | 30 | NA |
| vivo | - | 25 | - | NA |
| Huawei | - | 21 | - | 25.4 |
| Xiaomi | - | 25.2 | - | NA |
| ZTE | - | 26 | 29.21 | NA |
| Average | 21 | 26.2 | 29.6 | NA |

#### 5.2.2.12   Summary for 48GHz simulation results

| Parameter |  |  | LG Electronics R4-2215920 | vivo R4-2216128 | Huawei R4-2216245 | Xiaomi R4-2216350 R4-2219075 | ZTE R4-2216426 R4-2219595 | Nokia R4-2218326 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency |  |  | 48 GHz | 48 GHz | 48 GHz | 48 GHz | 48 GHz | 48 GHz |  |
| CBW |  |  | 100MHz | 50MHz/100MHz | 50MHz | 50MHz | 100MHz | 50MHz |  |
| SCS |  |  | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz |  |
| HARQ |  |  | none | none | none | none | none | none |  |
| RANK |  |  | RANK1 | RANK1 | RANK1 | RANK1 | RANK1 | RANK1 |  |
| Phase noise model |  |  | Option b): example2 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option d): example1 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option a): example1 (UE) + example1(BS) | Option d): example1 (UE) + example2(BS) |
| Tx EVM=Rx EVM |  |  | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% |
| Target SNR(dB) | TDL-A | MCS21 | NA | NA |  | NA | NA |  |  |
|  |  | MCS23 | NA | NA |  | NA | NA |  |  |
|  | TDL-D | MCS21 | NA | NA | 22.6 | NA | NA | NA | NA |
|  |  | MCS23 | NA | NA |  | NA | NA | NA | NA |
|  | AWGN | MCS21 | 27.5 | NA |  | 25 | NA |  |  |
|  |  | MCS23 | NA | NA |  | NA | NA |  |  |

### 5.2.3 Phase noise profiles evaluation

#### 5.2.3.1 Simulation assumptions

| Parameters | Value |
| --- | --- |
| Frequency | 29, 39 GHz |
| SCS | 60kHz, 120kHz |
| BW | 100MHz |
| RB Size | 128RB for 60kHz, 64RB for 120kHz |
| Background AWGN | No additional noise |
| Time offset/Frequency offset | 0 |
| Antenna configuration | 1T1R |
| Modulation | 256QAM |
| Waveform type | CP-OFDM / DFT-s-OFDM |
| DMRS | 3 symbols per slot (UL DMRS add-pos = 2) |
| PTRS configuration | OFF/ON For CP-OFDM: L-PTRS (Time density) = 1 (every 1 symbol) K-PTRS (Freq density) =2 (every 2 RBs) For DFTs-OFDM: L-PTRS (Time density) = 1 (every symbol) N_group = 8, N_samp = 4 Other PTRS configurations can be considered for narrow RB allocations, especially for DFT-s-OFDM. |
| EVM measurement | Data aided EVM calculation, based on ideal data signal |
| Phase noise profiles | Refer to clause 5.2.3.2 |

#### 5.2.3.2 Phase noise profiles

TBD

### 5.2.4	MPR simulation

#### 5.2.4.1   Simulation assumptions

Antenna configuration and PA calibration point for MPR simulation

      - The MPR evaluation was performed by using 32 PAs, 16 for each polarizations within an antenna array for PC1/2/5 keeping align with the antenna configuration agreed in system level simulation.

      - PA calibration point should follow current definition in Spec 38.101-2:

  - The waveform defined by BW = 100 MHz, SCS = 120 kHz, DFT-S-OFDM QPSK, 20RB23 is the reference waveform with 0 dB MPR and is used for the power class definition.

  - Calculate MPR as total backoff needed for 256QAM from this calibration point.

Emission requirements for MPR simulation

Follow the current Spec 38.101-2, otherwise specified

      - Occupied bandwidth (Table 6.5.1-1 in TS 38.101-2)

      - General NR spectrum emission mask for FR2-1 (Table 6.5.2.1-1 in TS 38.101-2)

      - NR ACLR1 for FR2-1 (Table 6.5.2.3-1 in TS 38.101-2)

      - General in-band emissions limit for FR2-1 (Tables 6.4.2.3.2-1 for PC1, 6.4.2.3.3-1 for PC2, 6.4.2.3.6-1 for PC5 in TS 38.101-2)

      - General NR spurious emission limits for FR2-1 (Table 6.5.3-2 in TS 38.101-2)

      - Maximum error vector magnitude (EVM) 3.5% for 29GHz and 39GHz as agreed in RAN4 #104-bis-e meeting.

EVM budget for MPR evaluation:

      - Only consider the total value of 3.5% for Tx EVM

  - Companies need to clarify the components of Tx EVM in their simulation results, including

    - Phase noise

    - Value for IQ imbalance

    - PA and transmitter non-linearity

#### 5.2.4.2   Simulation results from LGE

R4-2304634

Clarification of EVM budge, PA model, Phase noise profile, PTRS configuration for CPE correction:

EVM budget

- For CP-OFDM (L=1, K=2), (64 RB,120 kHz)

| Tx EVM contributor | EVM (%) | SNR (dB) |
| --- | --- | --- |
| Phase Noise+IQ Imbalance | 2.95 | 30.6 |
| PA Non-linearity & Transmitter | 1.88 | 34.5 |
| Total | 3.50 | 29.1 |

- For DFT-s-OFDM (no PTRS correction), (64 RB,120 kHz)

| Tx EVM contributor | EVM (%) | SNR (dB) |
| --- | --- | --- |
| Phase Noise+IQ Imbalance | 2.30 | 32.8 |
| PA Non-linearity & Transmitter | 2.64 | 31.6 |
| Total | 3.50 | 29.1 |

PA

- Using the 39 dBm PA model considering the Tx post loss for PC1

- Using the 27 dBm PA model considering the Tx post loss for PC2

Phase noise profile

- Using the example 1 in TR38.803.

PTRS configuration for CPE correction

- Using the PTRS (K=2, L=1) for CP-OFDM

- No PTRS for DFT-s-OFDM

Table 5.2.4.2-1 RB allocations

For PC1 ■Considered RB allocation for 100 MHz in FR2-1 256QAM MPR simulation ■Region 1 ●DFT-s-OFDM : [Start RB position:22, allocated RB: 20] ●CP-OFDM : [Start RB position:22, allocated RB: 22] ■Region 2 ●DFT-s-OFDM : [Start RB position:16, allocated RB: 32] ●CP-OFDM : [Start RB position:16, allocated RB: 32] ■Outer RB allocations ●DFT-s-OFDM : MAX([Start RB position:0, allocated RB: 64], [Start RB position:0, allocated RB: 10]) ●CP-OFDM : MAX([Start RB position:0, allocated RB: 66], [Start RB position:0, allocated RB: 10]) ■Considered RB allocation for 400 MHz in FR2-1 256QAM MPR simulation ■Region 1 ●DFT-s-OFDM : [Start RB position:60, allocated RB: 60] ●CP-OFDM : [Start RB position:66, allocated RB: 60] ■Region 2 ●DFT-s-OFDM : [Start RB position:60, allocated RB: 120] ●CP-OFDM : [Start RB position:66, allocated RB: 132] ■Outer RB allocations ●DFT-s-OFDM : MAX([Start RB position:0, allocated RB: 264], [Start RB position:0, allocated RB: 40]) ●CP-OFDM : MAX([Start RB position:0, allocated RB: 264], [Start RB position:0, allocated RB: 40]) For PC2 ■Considered RB allocation for 100 MHz in FR2-1 256QAM MPR simulation ■Region 1 inner RB allocation ●DFT-s-OFDM : [Start RB position:22, allocated RB: 20] ●CP-OFDM : [Start RB position:23, allocated RB: 20] ■Edge RB allocations ●DFT-s-OFDM : MAX([Start RB position:0, allocated RB: 64], [Start RB position:0, allocated RB: 1], [Start RB position:0, allocated RB: 20]) ●CP-OFDM : MAX([Start RB position:0, allocated RB: 66], [Start RB position:0, allocated RB: 1], [Start RB position:0, allocated RB: 20]) ■Considered RB allocation for 400 MHz in FR2-1 256QAM MPR simulation ■Region 1 inner RB allocation ●DFT-s-OFDM : [Start RB position:88, allocated RB: 80] ●CP-OFDM : [Start RB position:88, allocated RB: 88] ■Edge RB allocations ●DFT-s-OFDM : MAX([Start RB position:0, allocated RB: 240], [Start RB position:0, allocated RB: 1], [Start RB position:0, allocated RB: 80]) ●CP-OFDM : MAX([Start RB position:0, allocated RB: 264], [Start RB position:0, allocated RB: 1], [Start RB position:0, allocated RB: 80])

Table  5.2.4.2-2 Simulated MPR results for PC1 UL256QAM

| CBW (100 MHz) | Region 1 [22, 20] | Limiting factor | Region 2 [16, 32] | Limiting factor | Outter 1 [0, 64] | Limiting factor | Outter 2 [0, 10] | Limiting factor |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | 8.04 | EVM | 8.05 | EVM | 8.38 | EVM | 6.68 | SEM |
| CBW (100 MHz) | Region 1 [22, 22] | Limiting factor | Region 2 [16, 32] | Limiting factor | Outter 1 [0, 66] | Limiting factor | Outter 2 [0, 10] | Limiting factor |
| CP-OFDM | 10.44 | EVM | 9.86 | EVM | 10.31 | EVM | 9.01 | EVM |
| CBW (400 MHz) | Region 1 [60, 60] | Limiting factor | Region 2 [60, 120] | Limiting factor | Outter 1 [0, 264] | Limiting factor | Outter 2 [0, 40] | Limiting factor |
| DFT-s-OFDM | 8.44 | EVM | 9.28 | EVM | 9.40 | EVM | 8.44 | EVM |
| CBW (400 MHz) | Region 1 [66, 60] | Limiting factor | Region 2 [66, 132] | Limiting factor | Outter 1 [0, 264] | Limiting factor | Outter 2 [0, 40] | Limiting factor |
| CP-OFDM | 10.86 | EVM | 11.96 | EVM | 10.76 | EVM | 11.96 | EVM |

[start RB position, allocated RB number]

Table  5.2.4.2-3 Simulated MPR results for PC2 UL256QAM

| CBW (100 MHz) | Region 1 [22, 20] | Limiting factor | Outter1 [0, 64] | Limiting factor | Outter 2 [0, 1] | Limiting factor | Outter 3 [0, 20] | Limiting factor |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | 7.54 | EVM | 7.54 | EVM | 6.21 | EVM | 6.50 | EVM |
| CBW (100 MHz) | Region 1 [23, 20] | Limiting factor | Outter1 [0, 66] | Limiting factor | Outter 2 [0,1] | Limiting factor | Outter 3 [0, 20] | Limiting factor |
| CP-OFDM | 9.48 | EVM | 9.50 | EVM | 8.27 | EVM | 9.05 | EVM |
| CBW (400 MHz) | Region 1 [88, 80] | Limiting factor | Outter1 [0, 240] | Limiting factor | Outter 2 [0, 1] | Limiting factor | Outter 3 [0, 80] | Limiting factor |
| DFT-s-OFDM | 8.91 | EVM | 8.91 | EVM | 7.19 | EVM | 7.84 | EVM |
| CBW (400 MHz) | Region 1 [88, 88] | Limiting factor | Outter1 [0,264] | Limiting factor | Outter 2 [0,1] | Limiting factor | Outter 3 [0, 80] | Limiting factor |
| CP-OFDM | 10.96 | EVM | 10.84 | EVM | 9.04 | EVM | 10.09 | EVM |

[start RB position, allocated RB number]

Table  5.2.4.2-4 Proposed MPRWT for power class 1, BWchannel ≤ 200 MHz, FR2-1

| Modulation |  | MPRWT (dB), BWchannel ≤ 200 MHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |  |
|  |  |  | Region 1 | Region 2 |
| DFT-s-OFDM | 256 QAM | ≤ 10 | ≤ 9.5 | ≤ 9.5 |
| CP-OFDM | 256QAM | ≤ 12 | ≤ 12 | ≤ 11.5 |
| Note: 256 QAM MPRWT requirements can be applied for operating band n256, n258, n259 and n261. |  |  |  |  |

Table  5.2.4.2-5 Proposed MPRWT for power class 1, BWchannel = 400 MHz

| Modulation |  | MPRWT (dB), BWchannel = 400 MHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |  |
|  |  |  | Region 1 | Region 2 |
| DFT-s-OFDM | 256 QAM | ≤ 11 | ≤ 10 | ≤ 11 |
| CP-OFDM | 256 QAM | ≤ 13.5 | ≤ 12.5 | ≤ 13.5 |
| Note: 256 QAM MPRWT requirements can be applied for operating band n256, n258, n259 and n261. |  |  |  |  |

Table  5.2.4.2-6 Proposed MPRWT for power class 2, BWchannel ≤ 200 MHz, FR2-1

| Modulation |  | MPRWT, BWchannel ≤ 200 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | 256 QAM | ≤ 9 | ≤ 9 |
| CP-OFDM | 256 QAM | ≤ 11 | ≤ 11 |
| Note: 256 QAM MPRWT requirements can be applied for operating band n256, n258, n259 and n261. |  |  |  |

Table  5.2.4.2-7 Proposed MPRWT for power class 2, BWchannel = 400 MHz, FR2-1

| Modulation |  | MPRWT, BWchannel ≤ 400 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | 256 QAM | ≤ 10.5 | ≤ 10.5 |
| CP-OFDM | 256 QAM | ≤ 12.5 | ≤ 12.5 |
| Note: 256 QAM MPRWT requirements can be applied for operating band n256, n258, n259 and n261. |  |  |  |

## 5.3	EVM test

### 5.3.1	CPE compensation methods

To compensate CPE for 256QAM EVM testing, the DMRS based channel estimation and PTRS extraction shall be as below:

- The DMRS based channel estimate in the PTRS-ready EVM calculator shall utilize CPE-corrected DMRS symbols.

- The PTRS extraction and correction stage in the PTRS-ready EVM calculator is the final refinement of the received signal.

### 5.3.2	PTRS correction methods

- For CP-OFDM

  - PTRS correction is implemented by de-rotation of each sub carrier in an OFDM symbol. The de-rotation angle is estimated as the frequency domain average of the phase rotation of all the PTRS tones in the allocation.

- For DFT-s-OFDM:

  - PTRS correction is implemented by de-rotation of each time-domain symbol by the estimated instantaneous phase deviation.

  - The instantaneous phase deviation impacting a data symbol due to DUT phase noise is estimated by linearly interpolating between the phase deviations determined for the nearest neighbouring PTRS groups. The phase deviation for each PTRS group is determined as the time domain arithmetic mean phase deviation of all PTRS symbols in the group.

### 5.3.3	EVM calculation flow with PTRS

Figure 5.3.3-1 shows a generalized EVM calculator signal flow with PTRS processing. In the flow chart, for CP-OFDM, the DFT and IDFT blocks are replaced by unity blocks; For DFT-s-OFDM, all blocks pertaining to layer 2 signals are disabled due to RAN1 has not introduced rank 2 UL. And Figure 5.3.3-1 shall be included in the annex of 38.101-2 as normative content.

Figure5.3.3-1: A generalized 2-Rx EVM calculator with PTRS processing

## 5.3	Implementation impact for UE

## 5.4	Specification impact

## 5.5	Conclusion

# 6	Study for beam correspondence

## 6.1	General

3GPP defined 5G NR Frequency Range 2 bands with huge bandwidths which can cater 5G NR use cases requiring higher data rates. However, these bands are also subject to challenging propagating conditions such as high path loss, absorption from the environment, penetration losses to name a few. To overcome these, beam management procedures have been defined in 3GPP and beam correspondence is one of the main aspects of beam management. Beam correspondence is the ability of the UE to determine its uplink transmit beam based on downlink measurements on one or more Rx beams. UE beam correspondence functionality for RRC_CONNECTED, RRC_INACTIVE and initial access in IDLE have already been specified in the RAN1 and RAN2 specifications in Release 15 but FR2 UE beam correspondence requirements have not yet been defined for RRC_INACTIVE and initial access in IDLE.

Without UE beam correspondence requirements for initial access and RRC_INACTIVE it is not possible to ensure good UE Random access Msg1/Msg A performance and uplink coverage in FR2 deployments due to varying UE performances.

Rel-15 RRC_INACTIVE and Rel-17 Small data transmission (SDT) have a large potential in UE power efficiency, latency and signalling overhead reduction. RRC_INACTIVE allows for reduced latency and UE power saving, while SDT further enhances this for small data sessions. Considering that UE power savings are especially important for successful FR2 operations and good end-user experience, it would be important that the networks could efficiently utilize RRC_INACTIVE and Small Data Transmissions for FR2 as well.

Small data transmission procedures have been defined in 3GPP Rel-17 for SDT-RA for both two step and four step RA during RRC Inactive with a view of providing power saving opportunities and reducing overhead signalling. Without well performing UE beam correspondence support, wide usage of RRC_INACTIVE and Small Data Transmission may not be feasible in practical FR2 deployments. To enable efficient use of RRC_INACTIVE and Small Data Transmission in FR2 deployments to save UE power with reasonable latencies it is important to develop FR2 UE beam correspondence requirements for RRC_INACTIVE in Rel-18.

## 6.2	Implementation impact to UE

One of the main characteristics of 5G NR beam management procedures is using directive beams hence requiring beam alignment between the UE and the base station. In FR2, the UE uses the beam refinement procedure to have narrow highly directive beams towards the gNodeB thereby increasing the uplink coverage. During a Random access procedure, the UE could use both the rough and fine beams. An example of UE simulated at 28 GHz (band n257) with 1x4 array placed on the sides of a smart phone in free space (no user presence). We assume all the UE surroundings, UE model, and antenna model are the same, only change the beam width. The simulation results in Figure 6.2-1 and Figure 6.2-2 show the EIRP coverage curves (using different simulation assumptions, UE surroundings, UE model, and antenna model may cause different results).

Figure 6.2-1. EIPR spherical coverage for single and multiple broad beams (Nokia)

Figure 6.2-2: EIRP spherical coverage for envelope of narrow beams (Nokia)

In this example, the current requirements in RRC_CONNETED mode of 11.5 dBm EIRP at 50MHz CBW and at 50%-ile is represented with the grey box. In Figure 6.2-1, the blue curve, purple curve, and red curve represent the spherical coverage of a broad beam from one panel, the sum of spherical coverages of 2 broad beams from two 2 panels, and the sum of spherical coverages of 3 broad beams from two 3 panels. In Figure 6.2-2, the blue curve represents the envelope of narrow beams with a single panel with 4 elements active. The purple curve represents the sum of envelopes of narrow beams from two 2 panels, and each with 4 elements active. The red curve represents the sum of envelopes of narrow beams from two 3 panels, and each with 4 elements active.

As illustrated above, the spherical coverage at the 50%-ile is 16.5 – 19 dBm in Figure 6.2-1 and 21.4 – 24.2 dBm in Figure 6.2-2. The choice of using narrow beams and using broad beams has a big impact to the EIRP spherical coverage of a UE. We can also observe that the EIPR spherical coverage increases as the number of panels increases.

During random access procedure for Initial access or even handovers, cell reselection and other scenarios where Random access is required, the 3GPP standards do not mandate the beam type the UE used. This is up to UE implementation if a rough beam or a narrow beam is to be used and how to achieve beam refinement.

## 6.3	Specification Impact

### 6.3.1	RF

In RRC connected state, beam correspondence requirement is considered as fulfilled if the UE meets the corresponding minimum peak EIRP and spherical coverage requirement for that power class with its chosen UL beams with or without relying on uplink beam sweeping. Beam correspondence tolerance requirement could also be considered for UE supporting beam management relying on UL beam sweeping.

While in initial access and RRC_INACTIVE state, UL beam sweeping is not available. The focus is how to specify UE beam correspondence requirements for only SSB-based beam correspondence without UL beam sweeping as well as the implication to legacy UE capabilities.

RAN4 discussion is mainly focused on minimum peak EIRP and spherical coverage requirement for beam correspondence in initial access and RRC_INACTIVE state in R18. Since the UE behavior for beam management in initial access, RA-SDT and CG-SDT in RRC_INACTIVE state is the same, common set of beam correspondence requirement can be defined to well accommodate these scenarios. To save test effort, the beam correspondence requirement only needs to be tested for initial access based on Msg1 which is thought to be the most critical scenario.

- Minimum peak EIRP

Only SSB based beam correspondence requirement for minimum peak for RRC_CONNECTED state is based on fine beam implementation which is considered as too restrictive and not applicable for initial access from the perspective of addressing different beam type implementation.

Minimum peak EIRP will not be considered for initial access.

- Spherical coverage requirement at 50%-tile

TBA later

### 6.3.2	RRM

No RRM specification impact is identified.

## 6.4	Testing impact

Unlike traditional RF requirements defined in the RRC_CONNECTED state, beam correspondence for initial access needs to be verified in RRC_IDLE, and several potential testability issues were raised during the discussion. The following aspects of test related issues were identified:

- Verification in RRC_IDLE

In general, the power test needs to be averaged over a period, which requires the UE to remain in RRC_IDLE and multiple PRACH transmission need to be triggered, so testing mode including holding RAR should be considered. The accumulative period of measurement for PRACH transmission shall be at least 1 ms.

- Maximum output power for PRACH

To guarantee stable test results, the requirement is defined and verified under maximum output power (MOP). Since the open loop power control is used for PRACH, how to ensure that the UE reaches and maintains MOP is a problem that has to be faced. During the initial access, the UE measures the RSRP and estimates the PL based on the broadcasted DL RS power at first. The required power for PRACH transmission will be calculated based on the estimated PL and the target Rx power at gNB and compares with the Pcmax if configured. The power control for PRACH is defined in [1] as:

[dBm],

where  is a pathloss calculated by the UE in dB as referenceSignalPower – higher layer filtered RSRP in dBm, and referenceSignalPower is provided by ss-PBCH-BlockPower. The parameter  is the PRACH target reception power PREAMBLE_RECEIVED_TARGET_POWER provided by higher layers as specified in clause 5.1.3 of [2]:

The MAC entity shall, for each Random Access Preamble:

1>	if PREAMBLE_TRANSMISSION_COUNTER is greater than one; and

1>	if the notification of suspending power ramping counter has not been received from lower layers; and

1>	if LBT failure indication was not received from lower layers for the last Random Access Preamble transmission; and

1>	if SSB or CSI-RS selected is not changed from the selection in the last Random Access Preamble transmission:

2>	increment PREAMBLE_POWER_RAMPING_COUNTER by 1.

1>	select the value of DELTA_PREAMBLE according to clause 7.3;

1>	set PREAMBLE_RECEIVED_TARGET_POWER to preambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA;

UE can ramp up its power when RAR is held since it will trigger PRACH retransmission, and adjusting each individual RA response window may affect the overall test time. UE may also achieve MOP quickly by setting the preambleReceivedTargetPower properly with as few as possible PRACH re-transmission. Based on the analysis above, The PRACH can achieve MOP by holding RAR with well-defined parameters. In addition, during the initial access, UE might start transmitting preamble once it detects the SSB RSRP higher than preconfigured threshold on one RX beam, even if this RX beam is not the optimal one that UE could form, so the test function need to be considered. The details of test function and the details of the parameter configured in the test can be left to RAN5.

- Beam lock function

Currently, the beam lock function is only available in the RRC_CONNECTED state, and if the beam lock function is absent during the verification in RRC_IDLE, many complex issues will appear, e.g., when the RAR is held to trigger PRACH retransmission, UE may attempt other beams since the RAR is not received which may lead to unstable test. It also will be difficult to measure the θ (or φ) component EIRP under φ (or θ) link polarization without beam lock function. In [3], the LS from RAN5 concludes it is feasible to introduce beam lock function for RRC_IDLE, so the concern above can be solved. RAN4 concludes that dedicated beam lock function for RRC_IDLE state is needed, and the details of new beam lock function design can be left to RAN5.

- Impact of power control accuracy

TBA

## 6.5	Conclusion

Editor’s note:

# Annex <A> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-08 | RAN4 #104-e | R4-2212590 |  |  |  | TR skeleton | 0.0.1 |
| 2022-10 | RAN4 #104-bis-e | R4-2217730 |  |  |  | TP for TR 38.891 on link level simulation assumptions for FR2 UL 256QAM | 0.1.0 |
| 2022-11 | RAN4 #105 | R4-2219122 |  |  |  | TP for TR 38.891 on link level simulation results and system level simulation assumption for FR2 UL 256QAM | 0.2.0 |
| 2023-02 | RAN4 #106 | R4-2303490 |  |  |  | TP for TR 38.891 on simulation results and system level simulation assumptions for FR2 UL 256QAM | 0.3.0 |
| 2023-04 | RAN4 #106bis-e | R4-2305067 |  |  |  | TP for TR 38.891 on system level simulation results for FR2 UL 256QAM | 0.4.0 |
| 2023-04 | RAN4 #106bis-e | R4-2306602 |  |  |  | TP for TR 38.891: General and specification impact | 0.4.0 |
| 2023-05 | RAN4 #107 | R4-2308804 |  |  |  | TP for TR 38.891 on simulation assumptions for phase noise profiles evaluation and MPR simulation results for FR2 UL 256QAM | 0.5.0 |
| 2023-05 | RAN4 #107 | R4-2307231 |  |  |  | TP to TR 38.891: System level simulation results for FR2-1 UL 256QAM | 0.5.0 |
| 2023-05 | RAN4 #107 | R4-2310291 |  |  |  | TP for TR 38.891: General and Specification impact | 0.5.0 |
| 2023-05 | RAN4 #107 | R4-2310292 |  |  |  | TP for TR 38.891: Beam type and DRX implications | 0.5.0 |
| 2023-05 | RAN4 #107 | R4-2310293 |  |  |  | TP for test related issue of beam correspondence in initial access | 0.5.0 |