3GPP TR 21.917 V0.4.0 (2022-03)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Release 17 Description;

Summary of Rel-17 Work Items

(Release 17)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

Description, Summary, Release 17

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	 Abbreviations	7

4	Rel-17 Executive Summary	8

5	5G Services to "verticals"	8

5.1	Network and application enablement for verticals	8

5.2	Location and positioning	9

5.3	Railways	12

5.4	Support of Non-Public and Private Networks	12

5.5	Industrial IoT	13

5.6	Media production	17

5.7	Other services to verticals	17

6	5G access via Satellite/Non-Terrestrial (NTN) link	17

7	V2V/D2D/Sidelink related	18

8	Enhancements to network architecture and application enablement	24

8.1	Edge computing	24

8.2	Slicing	25

8.3	Access Traffic Steering, Switch and Splitting support in the 5G system architecture; Phase 2	26

8.4	Self-Organizing (SON)/Autonomous Network	26

8.5	Minimization of service Interruption	30

9	Multicast-Broadcast Services (MBS) and Mission Critical (MC)	30

10	Multi-(U)SIM	35

11	Messaging improvements	36

12	Drone/UAS/UAV/EAV identification	38

13	Authentication and key management for applications based on 3GPP credential in 5G	39

14	Multimedia Priority Service (MPS) Phase 2	39

15	NR enhancements	39

16	Other NR-only improvements	50

17	NR Carrier Aggregation	57

17.1	NR intra band Carrier Aggregation (itrabCA)	57

17.2	NR inter band Carrier Aggregation (iterbCA)	57

18	NR and LTE	58

19	NR new/modified bands	62

20	LTE new/modified bands	63

21	LTE inter-band Carrier Aggregation (LTE iterbCA)	64

22	User plane improvements	64

23	Signalling optimisations	66

24	Other Release 17 Features	69

25	Management-only Features	72

26	Features without summary	73

Annex A: List of expected contributions	74

Annex A: Template for contributing	86

Annex B: Process to get further information	87

B.1	General	87

B.2	Direct method	88

B.3	Step by step method	88

Annex C: Change history	91

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document provides a summary of each and every 3GPP Release 17 Feature, and more generally of all Work Items for which a summary has been agreed to be provided.

These summaries are based on the inputs issued by the Work Item Rapporteurs, sometimes modified by the Work Plan manager to ensure overall consistency of this document. The original inputs can be retrieved as temporary document (tdoc). The original tdoc number is provided just below the table of each clause.

The present document presents the "initial state" of the Features introduced in Release 17, i.e. as they are by the time of publication of this document. Each Feature is subject to be later modified or enhanced, over several years, by the means of Change Requests (CRs). To further outline a feature at a given time, it is recommended to retrieve all the CRs which relate to the given Feature, as explained in Annex C. As a first approach, the list of all the Change Requests related to a Work Item can be retrieved by selecting "TSG Status = Approved" in the Work Item’s page of the 3GPP Portal at:

https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem= [6-digit identifier of the Work Item, separated by a coma if several Work Items]

E.g. for “NR support for high speed train scenario”: the table at the beginning of its section indicates that the corresponding Work Items have the numbers 840192 and 840292. So, all the CRs related to this Feature are retrieved by selecting "TSG Status = Approved" in https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=840192,840292

Editor’s note: INPUTS ARE EXPECTED from rapporteurs. The list of expected contributors is provided at the beginning of each section. 
It also appears in “Annex B: List of contributors”, in the cell highlighted in yellow. Annex A provides the expected rapporteur in term of Working Group. E.g. if “SA2” is indicated, it means: “The SA2 WID rapporteur is expected to provide the summary for the full feature”.
The name of the rapporteur is provided in “Annex C: Full list of Rel-16 Work Items”, in the column “WI_rapporteur_e_mail”.

Editor’s note: to the contributors: 
The template to be used for the contribution is provided in “Annex A: Template for contributing”. The input type in 3GU is “WI Summary”.
The target audience of this document are readers inside and outside the 3GPP community. Contributions should be written as to be understandable by non-specialist. 
The summary should reflect the actual outcome of the Feature - and not the initial intentions given in the WID.
An exhaustive list of all the CRs and TS/TRs is not to be provided in the summary (instead, the procedure on how to retrieve this information is given in section 4). Only the essential CRs and TS/TRs can be mentioned in the Reference. 
When the usual Stage 1/Stage 2/Stage 3 process is not followed -and in particular if a full stage is missing- explain why.

# 1	Scope

The present document provides a summary of each Release 15 Feature or, whenever needed, of each significant Work Item.

The information provided in the present document is limited to an overview of each Feature, explaining briefly its purpose and the main lines of the system's behaviour to execute the Feature.

More information is available by consulting the 3GPP Ultimate web site, as explained in "Annex C: Process to get further information".

The present document presents the "initial state" of the Features introduced in Release 15, i.e. as they are by the time of publication of the present document. Each Feature is subject to be later modified or enhanced, over several years, by the means of Change Requests (CRs). It is therefore recommended to retrieve all the CRs which relate to the given Feature, as explained in Annex C, to further outline a feature at a given time.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	TR 21.905: "Vocabulary for 3GPP Specifications".

NOTE:	Due to the specificity of the present document, consisting in a collection of independent summaries, the references are given at the end of each clause rather than in this clause.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

Void.

## 3.3		Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1]. Abbreviations specific to a given clause are provided in the clause they appear.

KPI	Key Performance Indicator

Rel	3GPP Release

# 4	Rel-17 Executive Summary

Editor's note: To be provided later.

# 5	5G Services to "verticals"

## 5.1	Network and application enablement for verticals

| 920058 | Rel-17 Enhancements of 3GPP Northbound Interfaces and Application Layer APIs | NBI17 |  | CP-211197 | Yali Yan, Huawei |
| --- | --- | --- | --- | --- | --- |
| 920013 | CT1 aspects of NBI17 | NBI17 | C1 | CP-211197 | Yali Yan, Huawei |
| 920053 | CT3 aspects of NBI17 | NBI17 | C3 | CP-211197 | Yali Yan, Huawei |
| 920054 | CT4 aspects of NBI17 | NBI17 | C4 | CP-211197 | Yali Yan, Huawei |

Summary based on the input provided by Huawei in CP-220347.

The 3GPP Northbound Interfaces and APIs are specified in 3GPP in order to enable external entities and third party Application Servers/Functions to access a set of exposed 3GPP network services and capabilities in a secure and controlled manner. The 3GPP application layer APIs are defined by 3GPP so as to allow the support of various applications/services (e.g. V2X, UAS, EDGE, etc.) over 3GPP networks and ensure the efficient use and deployment of these applications/services over 3GPP systems via an optimized application layer framework.

3GPP northbound interfaces/APIs as well as application layer APIs, which are accessed by various external entities including third party Application Servers/Functions, should be continuously maintained and optimized. Therefore, the main work carried out was to introduce pure stage 3 (i.e. there are no related stage 1 nor stage 2 requirements) technical improvements and enhancements, i.e. improvement of the overall efficiency, reliability and flexibility, enhancement of the signalling efficiency, consolidation of the common protocol aspects, alignment with the 5GC service based principles and guidelines when relevant, corrections/changes missed in the previous 3GPP Releases, etc., especially that such enhancements may not be covered by the other more dedicated and functionality-driven work items.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=920058,920013,920053,920054

[1]	TS 29.122: "T8 reference point for Northbound APIs"

[2]	TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3"

[3]	TS 29.549: "Service Enabler Architecture Layer for Verticals (SEAL); Application Programming Interface (API) specification; Stage 3"

[4]	TS 29.558: "Enabling Edge Applications; Application Programming Interface (API) specification; Stage 3"

| 920012 | Enhancement of 5G PCC related services in Rel-17 | en5GPccSer17 | C3 | CP-211193 | Xiaoyun Zhou, Huawei |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Huawei in CP-211193.

The stage 3 procedures related to EPS functionality which are not completely covered in 5GS_Ph1-CT and en5GPccSer are defined.

The protocols and APIs for policy and charging control have been specified in the previous 3GPP Releases. When the 5GS was deployed, it was found that some functionalities defined in Gx/Rx for EPS are still applicable in the 5GS and EPS interworking scenario. There are also some aspects of 5GS that are not supported by the Rx interface when the Rx interworks with the 5GS.

In order to optimize the operator’s management, alignments have been made between the EPS and 5GS interfaces.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=920012

[1]		3GPP TS 29.512: "5G System; Session Management Policy Control Service ".

[2]		3GPP TS 29.513: "5G System; Policy and Charging Control signalling flows and QoS parameter mapping; Stage 3".

[3]		3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[4]		3GPP TS 29.214: "Policy and Charging Control over Rx reference point 5".

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 920049 | Enhanced Service Enabler Architecture Layer for Verticals | eSEAL |  | SP-200987 | Basavaraj (Basu) Pattan, Samsung |
| 900024 | Stage 2 of eSEAL | eSEAL | S6 | SP-200987 | Basavaraj (Basu) Pattan, Samsung |
| 920006 | CT1 aspects of eSEAL | eSEAL | C1 | CP-212098 | Sapan Shah (Samsung Electronics) |
| 920050 | CT3 aspects of eSEAL | eSEAL | C3 | CP-212098 | Sapan Shah (Samsung Electronics) |

## 5.2	Location and positioning

| 900060 | RAN aspects of NR positioning enhancements | NR_pos_enh |  | RP-210903 | Intel |
| --- | --- | --- | --- | --- | --- |
| 860034 | Study on NR positioning enhancements | FS_NR_pos_enh | R1 | RP-202094 | CATT |
| 900160 | Core part: NR positioning enhancements | NR_pos_enh-Core | R1 | RP-210903 | Intel |
| 900260 | Perf. part: NR positioning enhancements | NR_pos_enh-Perf | R4 | RP-210903 | Intel |

Summary based on the input provided by Intel in RP-220919.

This Work Item specifies solutions for NR Positioning enhancements, including improvement of positioning accuracy (Mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays, UL AOA for  network based positioning, DL-AOD for UE based and network based positioning and multipath/NLOS mitigation) and latency (Preconfigured MG/PPW, storing LPP capabilities in the AMF, preconfigured assistance data and scheduled location time) of Rel-16 NR positioning methods, improvements of network efficiency (On-Demand PRS transmission), improvement of device efficiency (positioning in RRC_INACTIVE), providing high integrity and reliability requirements (GNSS integrity) and enhancements of A-GNSS positioning (BDS B2a,/B3I and NavIC to NR). This completed normative work is a continuation of the 3GPP Rel.17 study item on NR Positioning enhancements in RP-202094.

Improvement of positioning accuracy

To improve positioning accuracy several solutions were considered:

Mitigation of gNB/UE Tx/Rx timing delay errors: For mitigation of gNB/UE Tx/Rx timing delay errors, multiple enhancements are introduced in Rel.17:

•	For DL-TDOA, a UE can be requested to provide the Rx TEG IDs together with RSTD measurements and a TRP can be requested to provide TRP Tx TEG association information of DL PRS resources to mitigate UE Rx timing errors and TRP Tx timing errors. A UE can also be requested to measure the same DL PRS resource of a TRP with different UE Rx TEGs and report the corresponding multiple RSTD measurements to mitigate UE Rx timing errors.

•	For UL-TDOA, Rel-17 supports a TRP to provide the Rx TEG IDs together with RTOA measurements and a UE to provide UE Tx TEG association information of UL positioning SRS resources to mitigate TRP Rx timing errors and UE Tx timing errors. A TRP can also be requested to measure the same UL SRS resource of a UE with different TRP Rx TEGs and report the corresponding multiple RTOA measurements to mitigate TRP Rx timing errors.

•	Multi-RTT, Rel-17 supports a UE/TRP to provide the RxTx TEG IDs, or combination of {Rx TEG ID, Tx TEG ID} together with UE/gNB Rx-Tx time difference measurements and a UE/gNB to provide its Tx TEG association information for mitigating UE/TRP Rx/Tx timing errors;

In addition, Rel-17 supports a UE to report more than one measurement instance of RSTD, DL RSRP, and/or UE Rx-Tx time difference measurements in a single measurement report, and support a TRP to report more than one measurement instance of RTOA, UL RSRP, and/or gNB Rx-Tx time difference measurements in a single measurement report. Each measurement instance is reported with its own timestamp;

UL-AOA enhancements: The new assistance information (expected UL-AOA value and uncertainty range) can be provided by the LMF to facilitate gNB measurements for NR UL-AOA, UL-TDOA and Multi-RTT positioning methods. Support of the first arrival path UL-AOA/ZOA measurement pair per SRS resource were introduced, including reporting of multiple per path AOA value pairs (including additional paths) to cope with possible ambiguity of angle measurements in antenna arrays with larger spacing than a half wavelength, and reporting ZoA only to cope with angle measurements in linear antenna arrays.

The per path UL SRS receive reference signal power (UL SRS-RSRPP) measurement definition was introduced.

The antenna reference point (ARP) location can be associated with UL measurements for NR Positioning (UL AOA, UL-RTOA, UL SRS-RSRP, UL SRS-RSRPP and gNB Rx-Tx time difference measurements).

Finally, to facilitate hybrid RAT dependent positioning, gNB can report to LMF the following set of measurements {one SRS-RSRP, multiple UL-AOAs (AOA/ZoA pairs), one UL-RTOA or one-gNB Rx-Tx time difference}.

DL-AOD enhancements: The enhancements of assistance data (a subset of PRS resources for each PRS resource for the purpose of prioritization of DL-AOD reporting or the boresight direction information for each PRS resource) were defined to improve DL-AOD estimation. In addition, the assistance information on DL-AOD/DL-AOA expected value and uncertainty range can be provided by LMF to UE. In Rel.17, for more accurate DL-AOD measurements, the gNB can provide the beam/antenna information to LMF (and LMF can further share it to UE for UE-based positioning). The per path DL PRS reference signal received power (DL PRS RSRPP) for the first path measurements were defined for DL-AOD estimation, and the maximum number of DL PRS RSRPP for the first path measurements was 24. Finally, the maximum number of DL PRS RSRP measurements per TRP was increased up to 24 compared to 8 in Rel.16.

Multi-path and NLOS mitigation: In Rel.17, multi-path (additional path) report enhancements and LOS/NLOS indication were introduced  for NR positioning solutions. The maximum number of additional paths that can be reported is increased (up to 8) with per path RSRP measurements and associated relative timing supported. Multiple UL-AOAs (up to 8) per additional path reporting is supported for the UL-TDOA and Multi-RTT positioning methods. The LOS/NLOS indicator was introduced that can be associated with specific measurements, DL/UL reference signals / resources for positioning.

Improvement of positioning latency

To improve the positioning latency, following solutions were considered:

Preconfigured measurement gap: To reduce latency of procedures for DL PRS processing with measurement gaps, the set of measurement gap patterns can be pre-configured to UE and activated/deactivated by gNB using new DL MAC CE signaling designed to control DL PRS measurement by UE. The UE can request activation and deactivation of the pre-configured MG using new UL MAC CE signaling introduced in Rel.17. The LMF can request activation of pre-configured MG using new NRPPa signaling introduced in Rel.17.

Preconfigured PRS processing window: To further reduce latency of DL PRS processing, UEs can perform DL PRS measurement outside measurement gaps and inside the active DL BWP with PRS having the same numerology as the active DL BWP. The gNB can use RRC signalling to pre-configure PRS processing window and DL MAC CE signaling for activation of PRS processing window, respectively. gNB can indicate the DL PRS processing priority relative to other DL signals/channels within the PRS processing window for PRS measurement outside MG.

M-sample measurement (M = 1): In Rel.17, LMF can request UE and/or TRP to perform measurement over either a single RS transmission period (M=1) or four RS transmission periods (M=4). Configuring M=1 for a UE reduces UE measurement period comparing to Rel.16 (In Rel-16, UE is expected to measure DL PRS over four periods (M = 4)). An AGC sample, in addition to the M=1 sample may be required at the UE subject to measurement conditions.

Lower Rx beam sweeping factor: In Rel.17, a new UE capability on lower Rx beam sweeping factor (<8) is introduced to reduce the PRS measurement latency for FR2 positioning frequency layers.

Storing LPP capability in AMF: The LMF may interact with the AMF to provide (updated) UE Positioning Capability to AMF and to receive stored UE Positioning Capability from AMF as described in TS 23.273. The LPP procedures to transfer UE LPP positioning capabilities may be skipped if the LMF already obtained the UE positioning capabilities from the AMF.

Preconfigured assistance data: Preconfigured assistance data is the DL-PRS assistance data (with associated validity criteria, i.e. area ID) that can be provided to the UE (before or during an ongoing LPP positioning session), to be then utilized for potential positioning measurements at a future time (e.g. for deferred MT-LR). Pre-configured DL-PRS assistance data may consist of multiple instances, where each instance is applicable to a different area within the network.

Scheduled location time: During positioning procedure, the LMF may obtain the scheduled location time from the AMF. Based on the obtained scheduled location time, the LMF may schedule location measurements by the UE and/or location measurement by the NG-RAN to occur at or near to the scheduled location time.

On-Demand PRS transmission

On-Demand PRS transmission procedure allows the LMF to control and decide whether PRS should be transmitted or not and whether the characteristics of an ongoing PRS transmission should be changed or not.

In case of UE-initiated On-Demand PRS, the LMF may configure the UE with pre-defined PRS configurations via LPP Provide Assistance Data message or via posSI. The UE sends an On-Demand PRS request to the LMF via LPP Request Assistance Data message. The On-Demand PRS request can be the request for a defined PRS configuration with PRS configuration ID or explicit parameter for PRS configuration and may be a request for PRS transmission or change to the PRS transmission characteristics for positioning measurements.

In case of LMF-initiated On-Demand PRS, the LMF and the UE may exchange LPP messages e.g., to obtain UE measurements or the DL-PRS positioning capabilities of the UE, etc).

The actual PRS changes are requested by the LMF irrespective of whether the procedure is UE- or LMF-initiated.

Positioning in RRC_INACTIVE state

Positioning may be performed when a UE is in RRC_INACTIVE. Any uplink LCS or LPP message can be transported in RRC_INACTIVE. If the UE initiated data transmission using UL SDT, the network can send DL LCS, LPP and RRC message (e.g. to configure SRS for UL positioning, if it is supported) to the UE. UE may also receive PRS or transmit SRS in RRC_INACTIVE. Support of all NR positioning measurements and support of NR positioning methods such as NR E-CID, DL-TDOA, DL-AOD, UL-AOA, UL-TDOA, Multi-RTT and RAT-independent positioning methods are extended for UEs in RRC_INACTIVE state.

GNSS Integrity

Positioning integrity is a measure of the trust in the accuracy of the position-related data and the ability to provide associated alerts. UE based GNSS integrity is supported in Rel-17. It allows the UE to determine and report the integrity results of the calculated location, where only protection level (PL) reporting mode is supported; the UE can use the integrity requirements and assistance data obtained via NG-RAN, together with its own measurements, to determine the integrity results of the calculated location.

For integrity operation, the network will ensure that:

P(Error > Bound for longer than TTA | NOT DNU) <= Residual Risk + IRallocation               (Equation 8.1.1a-1)

for all values of IRallocation in the range irMinimum <= IRallocation <= irMaximum

for all the errors in Table 8.1.2.1b-1 as specified in section  8.1.2.1b of TS38.305, which have corresponding integrity assistance data available and where the corresponding DNU flag(s) are set to false.

Enhancements of A-GNSS positioning

Enhancements of A-GNSS positioning specified: BDS B2a signal; BDS B3I signal; NavIC to NR.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=900060,860034,900160,900260

[1]	RP-220803 “Status Report to TSG on NR Positioning Enhancements”, Intel Corporation

[2]	3GPP TR 38. 857 “Study on NR positioning enhancements” v17.0.0

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 910052 | Enhancement to the 5GC LoCation Services-Phase 2 | 5G_eLCS_ph2 |  | SP-200082 | Ming Ai, CATT |
| 870001 | Stage 2 of 5G_eLCS_ph2 | 5G_eLCS_ph2 | S2 | SP-200082 | Ming Ai, CATT |
| 910006 | CT aspects of 5G_eLCS_ph2 | 5G_eLCS_ph2 | ct | CP-211090 | Chenxi Bao, CATT |
| 910053 | CT1 aspects of 5G_eLCS_ph2 | 5G_eLCS_ph2 | C1 | CP-211090 | Chenxi Bao, CATT |
| 910054 | CT3 aspects of 5G_eLCS_ph2 | 5G_eLCS_ph2 | C3 | CP-211090 | Chenxi Bao, CATT |
| 910055 | CT4 aspects of 5G_eLCS_ph2 | 5G_eLCS_ph2 | C4 | CP-211090 | Chenxi Bao, CATT |

Summary based on the input provided by XXX in xP-22xxxx.

## 5.3	Railways

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 840042 | Complete Gap Analysis for Railways Mobile Communication System | MONASTERYEND | S1 | SP-190312 | Gach, Guillaume, UIC |
| 890035 | Enhancements to Application Architecture for the Mobile Communication System for Railways Phase 2 | eMONASTERY2 |  | SP-191104 | Oettl, Martin, Nokia |
| 840037 | Stage 2 of eMONASTERY2 | eMONASTERY2 | S6 | SP-191104 | Oettl, Martin, Nokia |
| 890063 | Stage 3 of eMONASTERY2 | eMONASTERY2 | C1 | CP-202256 | Gkatzikis, Lazaros, Nokia(Nokia) |
| 840043 | Study on Future Railway Mobile Communication System3 | FS_FRMCS3 | S1 | SP-200578 | Gach, Guillaume, UIC, |

Summary based on the input provided by XXX in xP-22xxxx.

See also "Introduction of 900MHz NR band for Europe for Rail Mobile Radio (RMR)" and "Introduction of 1900MHz NR TDD band for Europe for Rail Mobile Radio (RMR)" under "NR enhancements" section

## 5.4	Support of Non-Public and Private Networks

| 890049 | Enhancement of Private Network support for NG-RAN | NG_RAN_PRN_enh | R3 | RP-212585 | China Telecom |
| --- | --- | --- | --- | --- | --- |
| 890149 | Core part: Enhancement of Private Network support for NG-RAN | NG_RAN_PRN_enh-Core | R3 | RP-212585 | China Telecom |

Summary based on the input provided by China Telecom in RP-220562.

This Work Item enables:

-	Support SNPN along with subscription / credentials owned by an entity separate from the SNPN including: broadcasting of information to enable SNPN selection for UEs with subscription/credentials owned by an entity separate from the SNPN [RAN2]; associated cell selection/reselection and connected mode mobility support [RAN2/RAN3]; necessary modifications over network interfaces (e.g. NG, Xn, F1, E1 etc) [RAN3]

-	Support UE onboarding and provisioning for NPN including: The UE onboarding relevant parameter broadcast from SIB [RAN2]; associated cell selection/reselection, cell access control and the connected mode mobility support [RAN2/RAN3]; necessary modifications over network interfaces (e.g. NG, Xn, F1, E1 etc) [RAN3]

-	Support of IMS voice and emergency services for SNPN [RAN2]: Broadcasting of relevant parameters [RAN2]

-	Support of PWS for SNPN [RAN2, RAN3]

Support SNPN along with subscription / credentials owned by an entity separate from the SNPN

For this feature, the term "Credentials Holder (CH)" is used for the external entity providing subscription or credential for SNPNs in the RAN2/RAN3 specifications and the term "Group IDs for Network Selection (GINs)" is used for the service provider Group IDs in the RAN2/RAN3 specifications.

In TS38.331, the following statement is added: “An SNPN may allow access to UEs being authorized using credentials or subscription owned by a separate credential holder (CH). The support of this feature is uniform across the SNPN as specified in TS 23.501 [3].”

For Uu interface, the indication of SNPN access with subscription of a Credentials Holder is broadcast in SIB1 per SNPN.

Optionally, Group IDs for Network selection (GINs) could be broadcast in SIBXY. Each GIN may be assigned to one or more SNPNs. There is a common list of GINs for both onboarding and SNPN access using external CHs.

For each SNPN there is a vector that describes which GINs are supported, the maximum number of GINs is 24 per cell.

For cell selection/reselection, since SA2 has the conclusion the indication of SNPN access with subscription of a Credentials Holder should be set uniformly, there is no impact on cell (re)selection to support SNPN with subscription or credentials by a separate entity.

No UE impact was identified on connected mode mobility for external CH.

Support UE onboarding and provisioning for NPN

For onboarding support, the section “Support of UE onboarding and remote provisioning” is added in TS 38.331.

The procedure of AMF selection in TS 38.410 is modified for supporting onboarding as follow:

“Therefore, a NAS node selection function is located in the NG-RAN node to determine the AMF association of the UE, based on the UE's temporary identifier, which was assigned to the UE by the AMF. When the UE's temporary identifier has not been yet assigned or is no longer valid the NG-RAN node may instead take into account other information (e.g. slicing information, onboarding indication) to determine the AMF.”

For Uu interface, the indication of UE onboarding and remote provisioning is broadcast in SIB1 per SNPN.

As for “on-boarding support” indicator, cell selection and suitability criteria of a SNPN cell are not affected. NAS will anyway allow access for onboarding only if the cell/SNPN supports onboarding.

Toggling the 1-bit onboarding indication in SIB1 allows to control congestion due to onboarding request.

No UE impact was identified on connected mode mobility for onboarding.

For NG interface, the new “Onboarding Support” IE is included within the same PLMN Support Item IE in the NG SETUP RESPONSE/AMF CONFIGURATION UPDATE message, the related statement for NG SETUP procedure, in 38.413, is added as follows:

“If the Onboarding Support IE is also included within the same PLMN Support Item IE, the NG-RAN node shall, if supported, consider that the AMF supports UE onboarding for the identified SNPN, as specified in TS 23.501 [9].”

Support of IMS voice and emergency services for SNPN

Emergency services are supported for SNPN in Rel-17, which is the new feature compared with Rel-16. UE in SNPN access mode could select the SNPN with imsEmergencySupportForSNPN=true for emergency services.

For Uu interface, the indication of support for emergency services is broadcast in SIB1 per SNPN.

Support of PWS for SNPN

PWS over SNPN is supported in Rel-17. In TS 38.300, the statement that ETWS and CMAS are not supported over SNPN is removed.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=890049,890149

[1]	RP-200732, Status report of WI: Private Network Support for NG-RAN; rapporteur: China Telecom.

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 840024 | Study on enhanced support of Non-Public Networks | FS_eNPN | S2 | SP-200094 | Peter Hedman |
| 910065 | Enhanced support of Non-Public Networks | eNPN |  | SP-200980 | Hedman, Peter, Ericsson |
| 900015 | Stage 2 for eNPN | eNPN | S2 | SP-200980 | Hedman, Peter, Ericsson |
| 910016 | CT aspects of eNPN | eNPN | ct | CP-212103 | Sedlacek, Ivo, Ericsson |
| 910066 | CT1 aspects of eNPN | eNPN | C1 | CP-212103 | Sedlacek, Ivo, Ericsson |
| 910067 | CT3 aspects of eNPN | eNPN | C3 | CP-212103 | Sedlacek, Ivo, Ericsson |
| 910068 | CT4 aspects of eNPN | eNPN | C4 | CP-212103 | Sedlacek, Ivo, Ericsson |
| 920025 | Security Aspects of eNPN | eNPN | S3 | SP-210422 | Jost, Christine, Ericsson |
| 880008 | Study on enhanced security support for Non-Public Networks | FS_eNPN_SEC | S3 | SP-200353 | Normann, Henrik, Ericsson |
| 870023 | Management of non-public networks (NPN) | OAM_NPN | S5 | SP-200189 | ZHANG, Kai, Huawei |

Summary based on the input provided by XXX in xP-22xxxx.

## 5.5	Industrial IoT

| 860051 | NR small data transmissions in INACTIVE state | NR_SmallData_INACTIVE |  | RP-212594 | ZTE |
| --- | --- | --- | --- | --- | --- |
| 860151 | Core part: NR small data transmissions in INACTIVE state | NR_SmallData_INACTIVE-Core | R2 | RP-212594 | ZTE |
| 860251 | Perf. part: NR small data transmissions in INACTIVE state | NR_SmallData_INACTIVE-Perf | R4 | RP-212594 | ZTE |

Summary based on the input provided by ZTE Corporation, Sanechips in RP-220154.

This work item enables the transmission of small signalling and/or data packets whilst the UE remains in RRC_INACTIVE state. Prior to Rel-17, NR supports RRC_INACTIVE state and UEs with infrequent (periodic and/or non-periodic) data transmission are generally maintained by the network in the RRC_INACTIVE state. Until Rel-16, the RRC_INACTIVE state doesn’t support data transmission. Hence, the UE has to resume the connection (i.e. move to RRC_CONNECTED state) for any DL (MT) and UL (MO) data. Connection setup and subsequently release to INACTIVE state happens for each data transmission however small and infrequent the data packets are. This results in unnecessary power consumption and signalling overhead.

Some examples of small and infrequent data traffic include Smartphone applications such as: traffic from Instant Messaging services (whatsapp, QQ, wechat  etc); Heart-beat/keep-alive traffic from IM/email clients and other apps; Push notifications from various applications. Other examples are non-smartphone applications such as: traffic from wearables (periodic positioning information etc); sensors (Industrial Wireless Sensor Networks transmitting temperature, pressure readings periodically or in an event triggered manner etc); smart meters and smart meter networks sending periodic meter readings.

As noted in 3GPP TS 22.891, the NR system shall be efficient and flexible for low throughput short data bursts, support efficient signalling mechanisms (e.g. signalling is less than payload) and reduce signalling overhead in general.

Signalling overhead from INACTIVE state UEs for small data packets is a general problem and will become a critical issue with more UEs in NR not only for network performance and efficiency but also for the UE battery performance. In general, any device that has intermittent small data packets in INACTIVE state will benefit from enabling small data transmission in INACTIVE.

The key enablers for small data transmission in NR, namely the INACTIVE state, 2-step, 4-step RACH and configured grant type-1 have already been specified as part of Rel-15 and Rel-16. So, this work builds on these building blocks to enable small data transmission in INACTIVE state for NR.

The Small Data Transmission (SDT) feature allows data and/or signalling transmission while the UE reamains in RRC_INACTIVE (i.e. without transitioning to RRC_CONNECTED state). SDT is enabled on a radio bearer basis and is initiated by the UE only if:

-	less than a configured amount of UL data awaits transmission across all radio bearers for which SDT is enabled, and;

-	the DL RSRP is above a configured threshold, and;

-	a valid SDT resource (either RACH or Configured grant) is available

SDT procedure is initiated with either a transmission over RACH (referred to as RA-SDT) or over Type 1 CG resources (referred to as CG-SDT). The SDT resources can be configured on initial BWP (for both RACH and CG). RACH and CG resources for SDT can be configured on either or both of NUL and SUL carriers. The initial PUSCH transmission during the SDT procedure includes at least the CCCH message. While the SDT procedure is ongoing, if data appears in a buffer of any radio bearer not enabled for SDT, the UE initiates a transmission of a non-SDT data arrival indication using UE assistance information message to the network and, if available, includes the resume cause. The network may configure UE to apply ROHC continuity for SDT either when the UE initiates SDT in the cell where the UE received RRCRelease and transitioned to RRC_INACTIVE state or when the UE initiates SDT in a cell of its RNA.

Details of RA-SDT

For RA-SDT, the network may configure 2-step and/or 4-step RA resources. The UE in RRC_INACTIVE initiates RACH and requests RRC resume together with UL SDT data/signalling. If the UE accesses a gNB other than the last serving gNB, the UL SDT data/signalling is buffered at the receiving gNB, and then the receiving gNB triggers the XnAP Retrieve UE Context procedure. RA-SDT is supported with and without UE context relocation and these two mechanisms as depicted in Figure 1 and Figure 2 below.

Figure 1: RA-SDT with UE context relocation			Figure 2: RA-SDT without UE context relocation

Details of CG-SDT

The CG-SDT resources are valid only within the cell the UE receives the previous RRCRelease (i.e. only for the no cell change case). When using CG resources for initial SDT transmission, the UE can perform autonomous retransmission of the initial transmission if the UE does not receive confirmation from the network. The network can schedule subsequent UL transmissions using dynamic grants or they can take place on the following CG resource occasions. The DL transmissions are scheduled using dynamic assignments. The UE can initiate subsequent UL transmission only after reception of confirmation for the initial PUSCH transmission from the network. For subsequent UL transmission, the UE cannot initiate re-transmission over a CG-SDT resource. CG-SDT can only be initiated with valid UL timing alignment. The UL timing alignment is maintained by the UE based on a SDT-specific timing alignment timer configured by the network via dedicated signalling and, for initial CG-SDT transmission, also by DL RSRP of configured number of highest ranked SSBs which are above a configured RSRP threshold. Upon expiry of the SDT-specific timing alignment timer, the CG resources are released.

References

List of related CRs:

select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=860051,860151,860251

[1]	RP-220153 	Status Report TSG for WI: NR small data transmissions in INACTIVE state

| 860044 | Additional enhancements for NB-IoT and LTE-MTC | NB_IOTenh4_LTE_eMTC6 |  | RP-211340 | Huawei |
| --- | --- | --- | --- | --- | --- |
| 860144 | Core part: Additional enhancements for NB-IoT and LTE-MTC | NB_IOTenh4_LTE_eMTC6-Core | R1 | RP-211340 | Huawei |
| 860244 | Perf. part: Additional enhancements for NB-IoT and LTE-MTC | NB_IOTenh4_LTE_eMTC6-Perf | R4 | RP-211340 | Huawei |

Summary based on the input provided by Huawei, HiSilicon in RP-220530.

This Rel-17 work item introduced additional enhancements for NB-IoT and LTE-MTC based on features standardized in Rel-13 and enhancements performed from Rel-14 through Rel-16. Rel-17 adds features such as 16QAM for NB-IoT in downlink and uplink, 14 HARQ processes in downlink for HD-FDD Cat. M1 UEs, NB-IoT neighbour cell measurement and triggering before RLF, NB-IoT carrier selection based on coverage level, and a maximum DL TBS of 1736 bits for HD-FDD Cat. M1 UEs [1].

16-QAM for unicast in UL and DL for NB-IoT

From Rel-13 to Rel-16, an NB-IoT UE can use QPSK for unicast NPDSCH, and QPSK or BPSK for unicast NPUSCH.

This feature allows an NB-IoT UE to use 16-QAM for unicast NPDSCH with TBS up to 4968 bits for standalone and guard-band deployments and 3624 bits for in-band deployments; and allows an NB-IoT UE to use 16-QAM for unicast NPUSCH with TBS up to 2536 bits (which can be transmitted with up to half the time-domain resources with respect to QPSK). When 16-QAM for unicast NPDSCH is configured, an NB-IoT UE can report the channel quality report by reporting the recommended NPDCCH repetition and NPDSCH modulation and coding scheme.

Additional PDSCH scheduling delay for 14-HARQ processes in DL for LTE-MTC

Figure 1: PDSCH transmission with 10 HARQ processes for HD-FDD Cat. M1 UEs

With 10 HARQ processes, a HD-FDD Cat. M1 UE cannot use all available downlink subframes to transmit PDSCH. As shown in Figure 1, the subframes #0, #1, #17, #18 cannot be scheduled to transmit PDSCH (as marked with an X).

This feature allows HD-FDD Cat. M1 UEs to use up to 14 HARQ processes in CE Mode A with an additional PDSCH scheduling delay to fully utilize the available BL/CE downlink and BL/CE uplink subframes, where the PDSCH scheduling delay can be indicated as 2 BL/CE DL subframes or a longer delay that consists of different subframe types. Two alternatives for the HARQ-ACK delay indication can be configured: either the HARQ-ACK delay consists of different subframe types, or the HARQ-ACK delay is indicated among sets of absolute subframes.

Neighbor cell measurements and measurement triggering before RLF for NB-IoT

This feature introduces measurements in RRC_CONNECTED for NB-IoT UEs to reduce the time taken for RRC connection reestablishment. The criteria to perform the measurements are signaled separately for intra- and inter-frequency measurements via broadcast signaling. Since dedicated measurements gaps are not supported, the UE may need to perform neighbour cell measurements during DL/UL idle periods that are provided by DRX or packet scheduling.

Carrier selection based on coverage level for NB-IoT

This feature introduces coverage-based paging in NB-IoT to reduce the latency and the resource usage in the network.

When coverage-based paging is enabled, up to two groups of paging carriers can be configured for lower levels of coverage enhancements. The eNB configures the UE during RRC connection release to use one of these groups of paging carriers.  If configured, the UE selects a paging carrier in its assigned group if its NRSRP is suitable according to the paging carrier configuration.  Coverage based paging is only applicable in the last cell where the coverage information was received.

Maximum DL TBS of 1736 bits for eMTC

From Rel-13 to Rel-16, the max DL TBS size for LTE-MTC Cat. M1 UEs is 1000 bits.

This feature allows HD-FDD Cat. M1 UEs to use a DL TBS of up to 1736 bits in CE Mode A, and the soft channel bits for UE supporting this feature is 43008 bits.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=860044,860144,860244

[1]	RP-220528	Status report for WI: Additional enhancements for NB-IoT and LTE-MTC; rapporteur: Huawei

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 850012 | Study on enhanced support of Industrial IoT | FS_IIoT | S2 | SP-200298 | Nokia, Devaki Chandramouli, |
| 880010 | Study on security for enhanced support of Industrial IoT | FS_IIoT_SEC | S3 | SP-200355 | Nokia, Anja Jerichow |
| 920024 | Security for enhanced support of Industrial IoT | IIoT_SEC | S3 | SP-210421 | Nokia, Anja Jerichow |
| 910059 | Support of Enhanced Industrial IIoT | IIoT |  | SP-200973 | Devaki Chandramouli, Nokia |
| 900008 | Stage 2 for IIoT | IIoT | S2 | SP-200973 | Devaki Chandramouli, Nokia |
| 910014 | CT aspects of support of enhanced IIoT | IIoT | ct | CP-212100 | Won, Sung Hwan, Nokia |
| 910060 | CT1 aspects of support of enhanced IIoT | IIoT | C1 | CP-212100 | Won, Sung Hwan, Nokia |
| 910061 | CT3 aspects of support of enhanced IIoT | IIoT | C3 | CP-212100 | Won, Sung Hwan, Nokia |
| 910062 | CT4 aspects of support of enhanced IIoT | IIoT | C4 | CP-212100 | Won, Sung Hwan, Nokia |
| 860045 | Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR | NR_IIOT_URLLC_enh |  | RP-210854 | Nokia |
| 860145 | Core part: Enhanced IoT and URLLC support for NR | NR_IIOT_URLLC_enh-Core | R2 | RP-210854 | Nokia |
| 860245 | Perf. Part: Enhanced IoT and URLLC support for NR | NR_IIOT_URLLC_enh-Perf | R4 | RP-210854 | Nokia |

Summary based on the input provided by XXX in xP-22xxxx.

## 5.6	Media production

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 840045 | Audio-Visual Service Production | AVPROD |  | SP-190304 | Wagdin, Ian, BBC |
| 800014 | Study on AVPROD | FS_AVPROD | S1 | SP-181015 | Roland Beutler, EBU |
| 840031 | Stage 1 of AVPROD | AVPROD | S1 | SP-191041 | Wagdin, Ian, BBC |
| 910001 | Study on Media Production over 5G NPN | FS_NPN4AVProd | S4 | SP-210241 | Lohmar, Thorsten, Ericsson |

Summary based on the input provided by XXX in xP-22xxxx.

See also "New bands and bandwidth allocation for 5G terrestrial broadcast - part 1"

## 5.7	Other services to verticals

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 840050 | Enhancements for cyber-physical control applications in vertical domains | eCAV |  | SP-190310 | Bahr, Michael, Siemens AG |
| 830020 | Study on eCAV | FS_eCAV | S1 | SP-190092 | Michael Bahr, Siemens AG |
| 840041 | Stage 1 of eCAV | eCAV | S1 | SP-191043 | Bahr, Michael, Siemens AG |

Summary based on the input provided by XXX in xP-22xxxx.

| 850046 | Asset Tracking for 5G | ATRAC |  | SP-190816 | Thierry Berisot, NOVAMINT |
| --- | --- | --- | --- | --- | --- |
| 810017 | Study on ATRAC | FS_5G_ATRAC | S1 | SP-180922 | Thierry Berisot, NOVAMINT |
| 850037 | Stage 1 of ATRAC | ATRAC | S1 | SP-190931 | Thierry Berisot, NOVAMINT |

| 840047 | Communication Service Requirements for Critical Medical Applications | CMED |  | SP-190306 | Lagrange, Mathieu, b<>com |
| --- | --- | --- | --- | --- | --- |
| 810016 | Study on CMED | FS_CMED | S1 | SP-180783 | Mathieu Lagrange, b<>com |
| 840033 | Stage 1 of CMED | CMED | S1 | SP-190306 | Lagrange, Mathieu, b<>com |

Summary based on the input provided by XXX in xP-22xxxx.

| 820025 | Study on application layer support for Factories of the Future in 5G network | FS_FFAPP | S6 | SP-200836 | Shao Weixiang, ZTE Corporation |
| --- | --- | --- | --- | --- | --- |
| 840025 | Study on enhancement of support for 5G LAN-type service | FS_5GLAN_enh | S2 | SP-190626 | Runze Zhou, Huawei |
| 900022 | Study on Charging Aspect of 5G LAN-type Services | FS_5GLAN_CH | S5 | SP-201081 | CHEN SHAN, Huawei |

# 6	5G access via Satellite/Non-Terrestrial (NTN) link

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 890034 | Integration of satellite components in the 5G architecture | 5GSAT_ARCH |  | SP-191335 | Cyril MICHEL, thales |
| 800048 | Stage 1 of 5GSAT | 5GSAT | S1 | SP-180326 | MICHEL, Cyril, Thales |
| 800026 | Study on architecture aspects for using satellite access in 5G | FS_5GSAT_ARCH | S2 | SP-181253 | MICHEL, Cyril |
| 860005 | (Stage 2 of) Integration of satellite components in the 5G architecture | 5GSAT_ARCH | S2 | SP-191335 | Cyril MICHEL, thales |
| 911030 | CT aspects of 5GC architecture for satellite networks | 5GSAT_ARCH-CT | ct | CP-210149 | Catovic, Amer, Qualcomm |
| 890005 | CT1 aspects of 5GC architecture for satellite networks | 5GSAT_ARCH-CT | C1 | CP-210149 | Catovic, Amer, Qualcomm |
| 920057 | CT3 aspects of 5GC architecture for satellite networks | 5GSAT_ARCH-CT | C3 | CP-210149 | Catovic, Amer, Qualcomm |
| 911031 | CT4 aspects of 5GC architecture for satellite networks | 5GSAT_ARCH-CT | C4 | CP-210149 | Catovic, Amer, Qualcomm |
| 930044 | CT6 aspects of 5GC architecture for satellite networks | 5GSAT_ARCH-CT | C6 | CP-210149 | Catovic, Amer, Qualcomm |
| 860046 | Solutions for NR to support non-terrestrial networks (NTN) | NR_NTN_solutions |  | RP-210908 | Thales |
| 860146 | Core part: Solutions for NR for NTN | NR_NTN_solutions-Core | R2 | RP-210908 | Thales |
| 860246 | Perf. Part: Solutions for NR for NTN | NR_NTN_solutions-Perf | R4 | RP-210908 | Thales |
| 920069 | NB-IoT/eMTC support for Non-Terrestrial Networks | LTE_NBIOT_eMTC_NTN | R1 | RP-211601 | MediaTek |
| 920169 | Core part: NB-IoT/eMTC support for Non-Terrestrial Networks | LTE_NBIOT_eMTC_NTN-Core | R1 | RP-211601 | MediaTek |
| 930019 | Architecture support for NB-IoT/eMTC Non-Terrestrial Networks in EPS | IoT_SAT_ARCH_EPS | S2 | SP-211124 | SEBIRE, Guillaume, MediaTek Inc. |
| 830025 | Study on management and orchestration aspects with integrated satellite components in a 5G network | FS_5GSAT_MO | S5 | SP-190138 | Floris Drijver, TNO |
| 860033 | Study on NB-IoT/eMTC support for NTN | FS_LTE_NBIOT_eMTC_NTN | R1 | RP-202689 | MediaTek |

Summary based on the input provided by XXX in xP-22xxxx.

# 7	V2V/D2D/Sidelink related

| 910037 | Support of advanced V2X services - Phase 2 | eV2XARC_Ph2 |  | SP-210090 | LaeYoung Kim, LG Electronics |
| --- | --- | --- | --- | --- | --- |
| 850013 | Study on V2X services – Phase 2 | FS_eV2XARC_Ph2 | S2 | SP-190631 | LaeYoung Kim, LG Electronics |
| 910021 | Stage 2 of eV2XARC_Ph2 | eV2XARC_Ph2 | S2 | SP-210090 | LaeYoung Kim, LG Electronics |
| 920005 | CT1 aspects of eV2XARC_Ph2 | eV2XARC_Ph2 | C1 | CP-211116 | Herrero Veron, Christian (Huawei) |
| 920047 | CT3 aspects of eV2XARC_Ph2 | eV2XARC_Ph2 | C3 | CP-211116 | Herrero Veron, Christian (Huawei) |
| 920048 | CT6 aspects of eV2XARC_Ph2 | eV2XARC_Ph2 | C6 | CP-211116 | Herrero Veron, Christian (Huawei) |

Summary based on the input provided by LG Electronics in SP-220277.

The study on the Phase 2 of the architecture enhancements for 3GPP support of advanced Vehicle-to-Everything (V2X) services (TR 23.776 [1]) concluded on the need for some normative work. This is what is specified by this work item, and deals with V2X communication over NR PC5 reference point (device-to-device) with power efficiency for pedestrian UEs, i.e. UEs for Vulnerable Road Users.

The support of QoS-aware NR PC5 power efficiency for pedestrian UEs is specified in TS 23.287 [2] as below:

-	Overall description about support of QoS-aware NR PC5 power efficiency for pedestrian UEs regarding NR PC5 Discontinuous Reception (DRX) operations.

-	PC5 DRX configuration, e.g. the mapping of PC5 QoS profile(s) to PC5 DRX cycle(s), default PC5 DRX configuration, for broadcast and groupcast when the UE is "not served by E-UTRA" and "not served by NR" as provisioned parameters for V2X communications over PC5 reference point.

-	NR Tx Profile for broadcast and groupcast as provisioned parameters for V2X communications over PC5 reference point.

For NR based unicast, groupcast and broadcast mode communication over PC5 reference point, PC5 DRX operations are supported to enable pedestrian UE power saving.

The V2X layer determines the respective V2X service types, and derives the corresponding PC5 QoS parameters based on either the mapping of V2X service types to PC5 QoS parameters, or the V2X Application Requirements for the V2X service type provided by the application layer. The V2X layer passes the PC5 QoS parameters and destination Layer-2 ID to the AS layer. For broadcast and groupcast, the V2X layer also determines the NR Tx Profile for the respective V2X service type based on the mapping of V2X service types to NR Tx Profiles and provides the NR Tx Profile to the AS layer.

When the PC5 DRX operation is needed, e.g. based on the NR Tx Profile in case of broadcast or groupcast, the AS layer determines the PC5 DRX parameter values for V2X communication over PC5 reference point, taking into account, e.g., PC5 QoS parameters and/or destination Layer-2 ID provided by the V2X layer.

For unicast, two UEs may negotiate the PC5 DRX configuration in the AS layer, and the PC5 DRX parameter values can be configured per pair of source and destination Layer-2 IDs and per direction in the AS layer.

For broadcast and groupcast when the UE is "not served by E-UTRA" and "not served by NR", the UE uses the provisioned PC5 DRX configuration for PC5 DRX operation.

Based on the Stage 2 requirements to support NR PC5 DRX operations, Stage 3 normative works are specified in TS 24.587 [3], TS 24.588 [4] and TS 31.102 [5].

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=910037,850013,910021,920005,920047,920048

[1]	TR 23.776: "Study on architecture enhancements for 3GPP support of advanced Vehicle-to-Everything (V2X) services; Phase 2".

[2]	TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[3]	TS 24.587: "Vehicle-to-Everything (V2X) services in 5G System (5GS); Stage 3".

[4]	TS 24.588: "Vehicle-to-Everything (V2X) services in 5G System (5GS); User Equipment (UE) policies; Stage 3".

[5]	TS 31.102: "Characteristics of the Universal Subscriber Identity Module (USIM) application".

| 860038 | Study on NR Sidelink relay | FS_NR_SL_relay | R2 | RP-202208 | OPPO |
| --- | --- | --- | --- | --- | --- |
| 911005 | NR Sidelink Relay | NR_SL_relay |  | RP-212819 | OPPO |
| 911105 | Core part: NR Sidelink Relay | NR_SL_relay-Core | R2 | RP-212819 | OPPO |
| 911205 | Perf. part: NR Sidelink Relay | NR_SL_relay-Perf | R4 | RP-212819 | OPPO |

Summary based on the input provided by OPPO, CMCC in RP-220211.

This WI specifies solutions to enable single-hop, sidelink-based, L2 and L3 based UE-to-Network (U2N) relay.

It specifies sidelink U2N relay supporting the following scenarios, i.e., for remote UE in and out of gNB coverage, in the same or different cell coverage as relay UE.

Figure 1: Scenarios for UE-to-Network Relay

Common aspect for both L2 and L3 U2N Relay

In order to enable remote UE and relay UE to identify each other and to establish sidelink connection, the scheme of sidelink discovery is introduced, including protocol stack design, interest report to network and etc. Further mechanism is adopted to enable network to configure the Uu RSRP threshold to (dis)allow remote / relay UE operation at specific cell location.

Figure 2: Protocol Stack of Discovery Message for UE-to-Network Relay

In order for remote UE to connection to the proper relay UE, relay (re)selection mechanism is introduced, in order for remote UE to base on the sidelink link quality to select proper relay UE. And relay UE can indicate the even of Uu link (e.g., Uu link disconnection or Uu link mobility) to remote UE, so that remote UE can decide whether to perform relay reselection.

In order to support PC5 radio resource control in NG-RAN, ProSe service authorisation information and PC5 QoS parameters for ProSe need to be made available in NG-RAN. Beside the authorization for 5G ProSe direct discovery and 5G ProSe direct communication, authorization IEs are introduced to indicate whether the UE is authorised to use a 5G ProSe Layer-3 and/or Layer-2 UE-to-Network Relay and 5G ProSe Layer-2 UE-to-Network Remote UE. 5G ProSe PC5 QoS parameters are also supported.

L2 U2N Relay specific aspect: User Plane

In order to support bearer mapping between sidelink connection between remote and relay UE, and Uu connection between relay UE and gNB, an adaptation layer is introduced, between RLC (which is per-hop deployed) and PDCP (which is end-to-end deployed). The header of adaptation layer would carry the identity for remote UE identification and bearer identification, in order for relay UE to perform packet forwarding between the two sides.

Figure 3: User plane protocol stack for L2 UE-to-Network Relay

Figure 4: Control plane protocol stack for L2 UE-to-Network Relay

L2 U2N Relay specific aspect: Control Plane

In order for remote UE to acquire system information and paging message via relay UE, the SIB forwarding mechanism is designed, so that relay UE can base on the request and detailed parameter (for paging reception) from remote UE to forward the necessary SIB and paging information to remote UE, upon acquisition of SIB and paging message from network. Furthermore, in order to save relay UE power consumption, network can use dedicated signalling to delivery paging message to relay UE if it is in RRC_CONNECTED state. Based on that, remote UE mobility in RRC_IDLE and RRC_INACTIVE state can be supported.

Furthermore, in order to support remote UE mobility in RRC_CONNECTED state, the switching between direct and indirect path for intra-gNB scenario is introduced. Via the newly introduced measurement event, remote UE can report the identified candidate connection (direct or indirect) to network, and network can correspondingly switch the UE to the connection (indirect or direct).

Figure 5: Procedure for U2N Remote UE switching to direct Uu cell

Figure 6: Procedure for U2N Remote UE switching to indirect path

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=860038,911005,911105,911205

[1]	RP-220210, Status report for WI on NR Sidelink Relay

| 860042 | NR Sidelink enhancement | NR_SL_enh |  | RP-202846 | LG Electronics |
| --- | --- | --- | --- | --- | --- |
| 860142 | Core part: NR Sidelink enhancement | NR_SL_enh-Core | R1 | RP-202846 | LG Electronics |
| 860242 | Perf. part: NR Sidelink enhancement | NR_SL_enh-Perf | R4 | RP-202846 | LG Electronics |

Summary based on the input provided by LG Electronics in RP-220521.

3GPP RAN technology for NR sidelink enhancement was specified through this WI to mainly define the means for power saving and enhanced reliability and reduced latency. This WI is the evolution of NR sidelink in Release 16.

This section provides a summary of the key functionalities of NR sidelink enhancement.

Power Savings Resource Allocation

The SL UE in Mode 2 can support partial sensing based resource allocation and random resource selection as power saving resource allocation methods. A SL mode 2 TX resource pool can be (pre)configured to enable full sensing only, partial sensing only, random selection only, or any combination(s) thereof. A UE decides which resource allocation scheme(s) can be used in the AS based on its capability (for a UE in RRC_IDLE/RRC_INACTIVE/OOC) and the allowed resource schemes in the resource pool configuration. Random resource selection is applicable to both periodic and aperiodic traffic.

A UE configured for partial sensing can perform periodic-based partial sensing and/or contiguous partial sensing for resource (re)selection. Periodic-based partial sensing can only be performed in a TX pool configured with partial sensing and periodic resource reservation. In periodic-based partial sensing, the UE monitors slots in periodic sensing occasion(s) for a given resource reservation periodicity. Contiguous partial sensing is performed by a UE configured for partial sensing when resource (re)selection is triggered by the UE in a TX pool configured with partial sensing. In contiguous partial sensing, the UE monitors slots in a contiguous sensing window which occur prior to the selected transmission resource.

Inter-UE Coordination (IUC)

The SL UE can support inter-UE coordination (IUC) in Mode 2, whereby a UE-A sends information about resources to UE-B, which UE-B then uses for resource (re)selection. The following schemes of inter-UE coordination are supported:

-	IUC scheme 1, where the coordination information sent from a UE-A to a UE-B is the preferred and/or non-preferred resources for UE-B’s transmission, and

-	IUC scheme 2, where the coordination information sent from a UE-A to a UE-B is the presence of expected/potential resource conflict on the resources indicated by UE-B’s SCI

In scheme 1, IUC can be triggered by an explicit request from UE-B, or by a condition at UE-A. UE-A determines the set of resources reserved by other UEs or slots where UE-A, when it is the intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half-duplex operation. UE-A uses these resources as the set of non-preferred resources, or excludes these resources to determine a set of preferred resources and sends the preferred/non-preferred resources to UE-B. UE-B’s resources for resource (re)selection can be based on both UE-B’s sensing results (if available) and the coordination information received from UE-A, or it can be based only on coordination information received from UE-A. For scheme 1, MAC CE and second-stage SCI or MAC CE only can be used to send IUC. The explicit request and reporting for IUC in unicast manner is supported.

In scheme 2, UE-A determines the expected/potential resource conflict within the resources indicated by UE-B’s SCI as either resources reserved by other UEs and identified by UE-A as fully/partially overlapping with the resources indicated by UE-B’s SCI, or as slots where UE-A is the intended receiver of UE-B and does not expect to perform SL reception on those slots due to half-duplex operation. UE-B uses the conflicting resources to determine the resources to be reselected and exclude the conflicting resources from the reselected resources.  For scheme 2, PSFCH is used to send IUC.

SL DRX

Sidelink supports SL DRX for unicast, groupcast, and broadcast. Similar parameters as defined for Uu (on-duration, inactivity-timer, retransmission-timer, cycle) are defined for SL to determine the SL active time for SL DRX. During the SL active time, the UE performs SCI monitoring for data reception (i.e., PSCCH and 2nd stage SCI on PSSCH). The UE may skip monitoring of SCI for data reception during SL DRX inactive time. The SL active time of the RX UE includes the time in which any of its applicable SL on-duration timer(s), SL inactivity-timer(s) or SL retransmission timer(s) (for any of unicast, groupcast, or broadcast) are running. In addition, the slots associated with announced periodic transmissions by the TX UE and the time in which a UE is expecting CSI report following a CSI request (for unicast) are considered as SL active time of the RX UE. When data is available for transmission to one or more RX UE(s) configured with SL DRX, the TX UE selects resources taking into account the active time of the RX UE(s) determined by the timers maintained at the TX UE.

For unicast, SL DRX is configured per pair of source L2 ID and destination L2 ID. The UE maintains a set of SL DRX timers for each direction per pair of source L2 ID and destination L2 ID. The SL DRX configuration for a pair of source/destination L2 IDs for a direction may be negotiated between the UEs in the AS layer. For SL DRX configuration of each direction, where one UE is the TX UE and the other is the RX UE. RX UE may send assistance information, which includes its desired on duration timer, SL DRX start offset, and SL DRX cycle,  to the TX UE and the mode 2 TX UE may use it to determine the SL DRX configuration for the RX UE. Regardless of whether assistance information is provided or not, the TX UE in RRC_IDLE/RRC_INACTIVE/OOC, or in RRC_CONNECTED and using mode 2 resource allocation, determines the SL DRX Configuration for the RX UE. For a TX UE in RRC_CONNECTED and using mode 1 resource allocation, the SL DRX configuration for the RX UE is determined by the serving gNB of the TX UE. TX UE sends the SL DRX configuration to be used by the RX UE to the RX UE. The RX UE may accept or reject the SL DRX configuration. A default SL DRX configuration for groupcast/broadcast can be used for DCR messages. When the TX UE is in RRC_CONNECTED, the TX UE may report the received assistance information to its serving gNB and sends the SL DRX configuration to the RX UE upon receiving the SL DRX configuration in dedicated RRC signaling from the gNB. When the RX UE is in RRC_CONNECTED, the RX UE can report the received SL DRX configuration to its serving gNB, e.g. for alignment of the Uu and SL DRX configurations. SL on-duration timer, SL inactivity-timer, SL HARQ RTT timer, and SL HARQ retransmission timer are supported in unicast. SL HARQ RTT timer and SL HARQ retransmission timer are maintained per SL process at the RX UE.  In addition to (pre)configured values for each of these timers, SL HARQ RTT timer value can be derived from the retransmission resource timing when SCI indicates more than one transmission resource. SL DRX MAC CE is introduced for SL DRX operation in unicast only.

For groupcast/broadcast, SL DRX is configured commonly among multiple UEs based on QoS profile and Destination L2 ID. Multiple SL DRX configurations can be supported for each of groupcast/broadcast. SL on-duration timer, SL inactivity-timer, SL HARQ RTT and SL retransmission timers are supported for groupcast. Only SL on-duration timer is supported for broadcast. SL DRX cycle, SL on-duration, and SL inactivity timer (only for groupcast) are configured per QoS profile. The starting offset and slot offset of the SL DRX cycle is determined based on the destination L2 ID. The SL HARQ RTT timer (only for groupcast) and SL HARQ retransmission timer (only for groupcast) are not configured per QoS profile or per destination L2 ID. For groupcast, the RX UE maintains an SL inactivity timer for each destination L2 ID, and selects the largest SL inactivity timer value if multiple SL inactivity timer values associated with different QoS profiles are configured for that L2 ID. For groupcast and broadcast, the RX UE maintains a single SL DRX cycle (selected as the smallest SL DRX cycle of any QoS profile of that L2 ID) and single SL on-duration (selected as the largest SL on-duration of any QoS profile of that L2 ID) for each destination L2 ID when multiple QoS profiles are configured for that L2 ID. For groupcast, SL HARQ RTT timer and SL retransmission timer are maintained per SL process at the RX UE. SL HARQ RTT timer can be set to different values to support both HARQ enabled and HARQ disabled transmissions. A default SL DRX configuration, common between groupcast and broadcast, can be used for a QoS profile which is not mapped onto any non-default SL DRX configuration(s). For groupcast, the TX UE restarts its timer corresponding to the SL inactivity timer for the destination L2 ID (used for determining the allowable transmission time) upon reception of new data with the same destination L2 ID. TX profile is introduced to ensure compatibility for groupcast and broadcast transmissions between UEs supporting/not-supporting SL DRX functionality. A TX profile is provided by upper layers to AS layer and identifies one or more sidelink feature group(s). A TX UE only assumes SL DRX for the RX UEs when the associated TX profile corresponds to support of SL DRX. A RX UE determines that SL DRX is used if all destination L2 IDs of interest have an associated TX profile corresponding to the support of SL DRX.

Alignment of Uu DRX and SL DRX for a UE in RRC_CONNECTED is supported for unicast, groupcast, and broadcast. Alignment of Uu DRX and SL DRX at the same UE is supported. In addition, for mode 1 scheduling, the alignment of Uu DRX of the TX UE and SL DRX of the RX UE is supported. For SL RX UEs in RRC_CONNECTED, alignment is achieved by the gNB.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=860042,860142,860242

[1]	Last status report: RP-220520

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 840035 | Study on enhancements to application layer support for V2X services | FS_eV2XAPP | S6 | SP-200110 | Niranth Amogh, Huawei Telecommunications India |
| 910075 | Enhanced application layer support for V2X services | eV2XAPP |  | SP-200831 | Niranth Amogh, Huawei Telecommunications India |
| 890036 | Stage 2 of eV2XAPP | eV2XAPP | S6 | SP-200831 | Niranth Amogh, Huawei Telecommunications India |
| 910019 | CT aspects of eV2XAPP | eV2XAPP | ct | CP-211109 | Herrero Veron, Christian (Huawei) |
| 910076 | CT1 aspects of eV2XAPP | eV2XAPP | C1 | CP-211109 | Herrero Veron, Christian (Huawei) |
| 910077 | CT3 aspects of eV2XAPP | eV2XAPP | C3 | CP-211109 | Herrero Veron, Christian (Huawei) |
| 900030 | Proximity based Services in 5GS | 5G_ProSe |  | SP-200972 | TBD |
| 830033 | Study on System enhancement for Proximity based Services in 5GS | FS_5G_ProSe | S2 | SP-190443 | Qiang Deng, CATT |
| 900007 | Stage 2 for Proximity based Services in 5GS | 5G_ProSe | S2 | SP-201096 | Qiang Deng, CATT |
| 910018 | CT aspects of proximity based services in 5GS | 5G_ProSe | ct | CP-212105 | Yong Jiang, CATT |
| 910072 | CT1 aspects of 5G_ProSe | 5G_ProSe | C1 | CP-212105 | Yong Jiang, CATT |
| 910073 | CT3 aspects of 5G_ProSe | 5G_ProSe | C3 | CP-212105 | Yong Jiang, CATT |
| 910074 | CT4 aspects of 5G_ProSe | 5G_ProSe | C4 | CP-212105 | Yong Jiang, CATT |
| 920081 | CT6 aspects of 5G_ProSe | 5G_ProSe | C6 | CP-212105 | Yong Jiang, CATT |
| 890018 | Study on charging aspects of Proximity-based Services in 5GS | FS_5G_Prose_CH | S5 | SP-200767 | Shu, Min, CATT |
| 880005 | Study on Security Aspects of Enhancement for Proximity Based Services in 5GS | FS_5G_ProSe_Sec | S3 | SP-200350 | Wei Zhou, CATT |
| 930008 | Security Aspects of Proximity based Services in 5GS | 5G_ProSe | S3 | SP-211120 | Wei Zhou, CATT |

Summary based on the input provided by XXX in xP-22xxxx.

# 8	Enhancements to network architecture and application enablement

## 8.1	Edge computing

| 920019 | Edge Computing Management | ECM | S5 | SP-210388 | Deepanshu Gautam, Samsung |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Samsung in SP-220020.

The ECM Work Item and the resulting specification in TS 28.538[4] provides management provisions and solutions for edge computing considering related requirements from SA6 EDGEAPP WI including lifecycle management, provisioning, performance assurance and fault supervision for EDGEAPP defined entities.

Edge computing is a well-known industry concept, with 3GPP 5G System Architecture supporting Edge computing deployments by enabling certain features as listed in sub clause 5.13 of TS 23.501[1]. SA6, as part of TS 23.558[2], have defined the functionality and concepts required for enabling Edge Applications in 3GPP networks. The functionalities defined to be deployed in 3GPP networks include Edge Application, Edge Enabler Server and Edge Data Network Configuration Server. The management provisions for these functionalities are addressed as part of this WI. The management solutions in preview of this WI includes.

| - Provisioning and LCM: Provisioning includes configuration and lifecycle management. It deals with defining NRMs (Network Resource Model) for the edge entities to be managed and also defining usage of those NRMs by the management services to achieve provisioning and lifecycle management. - Performance Assurance: It deals with defining edge specific performance measurements and KPIs. The collection mechanism for the same is also defined. - Fault Supervision: It deals with defining edge specific alarm. The collection mechanism for the same is also defined. - Connection to 5GC: Connecting edge nodes to 5GC is crucial part of edge computing management. The defined NRM fragments to enable the same. |  |
| --- | --- |

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=920019

[1]	TS 23.501: “System architecture for the 5G System (5GS)”

[2]	TS 23.558: “Architecture for enabling Edge Applications”

[3]	TR 28.814: “Study on enhancements of edge computing management”

[4]	TS 28.538: “Edge Computing Management”

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 880042 | Architecture for enabling Edge Applications | EDGEAPP |  | SP-200109 | Gupta, Nishant, Samsung |
| 830008 | Study on Application Architecture for enabling Edge Applications | FS_EDGEAPP | S6 | SP-190065 | Nishant Gupta, Samsung |
| 860006 | Architecture for enabling Edge Applications | EDGEAPP | S6 | SP-200886 | Gupta, Nishant, Samsung |
| 900006 | CT aspects for Enabling Edge Applications | EDGEAPP |  | CP-211196 | Narendranath Durga Tangudu |
| 900034 | CT1 aspects for Enabling Edge Applications | EDGEAPP | C1 | CP-211196 | Narendranath Durga Tangudu |
| 900035 | CT3 aspects for Enabling Edge Applications | EDGEAPP | C3 | CP-211196 | Narendranath Durga Tangudu |
| 830032 | Study on enhancement of support for Edge Computing in 5GC | FS_enh_EC | S2 | SP-200093 | Hui Ni, Huawei |
| 910048 | Enhancement of support for Edge Computing in 5G Core network | eEDGE_5GC |  | SP-201107 | Hui Ni, Huawei Technologies |
| 900016 | Stage 2 of eEDGE_5GC | eEDGE_5GC | S2 | SP-201107 | Hui Ni, Huawei Technologies |
| 910005 | CT aspects of 5G eEDGE | eEDGE_5GC | ct | CP-212021 | Qi Caixia, Huawei |
| 910049 | CT1 aspects of 5G eEDGE | eEDGE_5GC | C1 | CP-212021 | Qi Caixia, Huawei |
| 910050 | CT3 aspects of 5G eEDGE | eEDGE_5GC | C3 | CP-212021 | Qi Caixia, Huawei |
| 910051 | CT4 aspects of 5G eEDGE | eEDGE_5GC | C4 | CP-212021 | Qi Caixia, Huawei |
| 920026 | Security Aspects of Enhancements of Support for Edge Computing in 5GC | eEDGE_5GC | S3 | SP-210423 | Bo Zhang, Huawei Technologies |
| 880002 | Study on Security Aspects of Enhancement of Support for Edge Computing in 5GC | FS_eEDGE_Sec | S3 | SP-200347 | Bo Zhang, Huawei Technologies |
| 930034 | Charging aspects of Edge Computing | EDGE_CH | S5 | SP-210861 | Yao, Yizhi, Intel |
| 880030 | Study on charging aspects of Edge Computing | FS_EDGE_CH | S5 | SP-200467 | Yizhi Yao, Intel |
| 930011 | Charging aspects of Edge Computing | EDGE_CH | S5 | SP-210861 | Yao, Yizhi, Intel |
| 870029 | Study on enhancements of edge computing management | FS_eEDGE_Mgt | S5 | SP-200195 | Joey Chou, Intel |

Summary based on the input provided by XXX in xP-22xxxx.

## 8.2	Slicing

| 9500xx | Network Slice charging based on 5G Data Connectivity | NETSLICE_DC_CH | S5 | SP-220158 | MATRIXX Software, Gerald Görmer |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by MATRIXX Software in SP-220075.

In 5G system when Network Slices allocated to third party providers are deployed by Mobile Network Operators, how these third-party providers can be charged for usage of assigned Network Slice(s), is not described. This work item introduces a description focusing on one particular type of Network Slice usage, leveraging from capabilities specified from Rel-15.

5G Data Connectivity charging specified in 5GS between SMF and CHF for individual UEs PDU sessions from Rel-15, is used with Converged Charging System (CCS) hosting the CHF extended to also cover the third-party provider (addressed under the tenant concept).

By receiving the S-NSSAI for each UE PDU session, the enhanced CCS is able to perform per tenant charging based on tenant's Network Slice total UEs data connectivity usage, the Network Slice being identified by the S-NSSAI.

The internal behaviour of the enhanced CCS is not specified in this release.

References

List of related CRs: SP-220157: "Introduction of Annex on Network slice charging"

[1]	TS 32.255: "Telecommunication management; Charging management; 5G Data connectivity domain charging; stage 2".

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 900032 | Enhancement of Network Slicing Phase 2 | eNS_Ph2 |  | SP-200976 | ZTE, Jinguo Zhu |
| 850010 | Study on Enhancement of Network Slicing Phase 2 | FS_eNS_Ph2 | S2 | SP-190931 | So, Tricci, ZTE |
| 900011 | Stage 2 for Enhancement of Network Slicing Phase 2 | eNS_Ph2 | S2 | SP-200976 | ZTE, Jinguo Zhu |
| 920059 | Stage 3 for Enhancement of Network Slicing Phase 2 | eNS_Ph2 | ct | CP-211091 | Hannah Wang, ZTE |
| 910041 | CT1 aspects of eNS_Ph2 | eNS_Ph2 | C1 | CP-211091 | Hannah Wang, ZTE |
| 910042 | CT3 aspects of eNS_Ph2 | eNS_Ph2 | C3 | CP-211091 | Hannah Wang, ZTE |
| 910043 | CT4 aspects of eNS_Ph2 | eNS_Ph2 | C4 | CP-211091 | Hannah Wang, ZTE |
| 911007 | Enhancement of RAN slicing for NR | NR_slice | R2 | RP-212534 | CMCC |
| 911107 | Core part: Enhancement of RAN slicing for NR | NR_slice-Core | R2 | RP-212534 | CMCC |
| 860022 | Study on network slice management enhancement | FS_NSMEN | S5 | SP-200766 | Brendan Hassett, Huawei Technologies Sweden AB, |
| 900023 | Study on Charging Aspects for Network Slicing Phase 2 | FS_NETSLICE_CH_Ph2 | S5 | SP-201082 | Matrixx, Gerald Görmer |
| 910023 | Study on enhanced security for network slicing Phase 2 | FS_eNS2_SEC | S3 | SP-210106 | Zander Lei, Huawei |
| 910026 | Study on network slice management capability exposure | FS_NSCE | S5 | SP-210131 | Xiaobo Yu, Alibaba Group |
| 910095 | Study on 5G NR UE full stack testing for Network Slicing | FS_NR_Slice_Test | R5 | RP-211977 | CMCC |

Summary based on the input provided by XXX in xP-22xxxx.

## 8.3	Access Traffic Steering, Switch and Splitting support in the 5G system architecture; Phase 2

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 900033 | Access Traffic Steering, Switch and Splitting support in the 5G system architecture; Phase 2 | ATSSS_Ph2 |  | SP-200977 | Apostolis Salkintzis, Lenovo |
| 840084 | Study on ATSSS_Ph2 | FS_ATSSS_Ph2 | S2 | SP-200095 | So, Tricci, ZTE |
| 900012 | Stage 2 of ATSSS_Ph2 | ATSSS_Ph2 | S2 | SP-200977 | Apostolis Salkintzis, Lenovo |
| 910013 | CT aspects of ATSSS_Ph2 | ATSSS_Ph2 | ct | CP-210136 | ZHOU Xingyue (Joy), ZTE |
| 910056 | CT1 aspects of ATSSS_Ph2 | ATSSS_Ph2 | C1 | CP-210136 | ZHOU Xingyue (Joy), ZTE |
| 910057 | CT3 aspects of ATSSS_Ph2 | ATSSS_Ph2 | C3 | CP-210136 | ZHOU Xingyue (Joy), ZTE |
| 910058 | CT4 aspects of ATSSS_Ph2 | ATSSS_Ph2 | C4 | CP-210136 | ZHOU Xingyue (Joy), ZTE |

Summary based on the input provided by XXX in xP-22xxxx.

## 8.4	Self-Organizing (SON)/Autonomous Network

| 860053 | Enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR and EN-DC | NR_ENDC_SON_MDT_enh |  | RP-193255 | CMCC |
| --- | --- | --- | --- | --- | --- |
| 860153 | Core part: Enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR and EN-DC | NR_ENDC_SON_MDT_enh-Core | R3 | RP-193255 | CMCC |

Summary based on the input provided by CMCC in RP-220822.

his work item introduces enhancement of SON and MDT features support in NR standalone and MR-DC, including

CCO, inter-system inter-RAT energy saving, inter-system load balancing, 2-step RACH optimization, mobility enhancement optimization, PCI selection, energy efficiency (OAM requirements), Successful Handovers Reports, UE history information in EN-DC, load balancing enhancement, MRO for SN change failure, RACH Optimisation enhancements, MDT enhancement and L2 measurements.

The key functionalities of this WI are described as below.

NR Coverage and Capacity Optimization (CCO)

NR Coverage and Capacity Optimization (CCO) function is to detect and mitigate coverage and cell edge interference issues.  Each NG-RAN node may be configured with alternative coverage configurations by OAM. The alternative coverage configurations contain relevant radio parameters and may also include a range for how each parameter is allowed to be adjusted. An NG-RAN node may autonomously adjust within and switch between coverage configurations. When a change is executed, a NG-RAN node may notify its neighbour NG-RAN nodes using the NG-RAN NODE CONFIGURATION UPDATE message with the list of cells and SSBs with modified coverage included. The list contains the CGI of each modified cell with its coverage state indicator and optionally the SSB index of each modified SSB with its coverage state indicator.

Inter-system inter-RAT energy saving

The solution builds upon the possibility for the NG-RAN node owning a capacity booster cell to autonomously decide to switch-off such cell to dormant state. The decision is typically based on cell load information, consistently with configured information. The switch-off decision may also be taken by O&M. The NG-RAN node indicates the switch-off action to the eNB over NG interface and S1 interface. The NG-RAN node could also indicates the switch-on action to the eNB over NG interface and S1 interface.

The eNB providing basic coverage may request a NG-RAN node’s cell re-activation based on its own cell load information or neighbour cell load information, the switch-on decision may also be taken by O&M. The eNB requests a NG-RAN node’s cell re-activation and receives the NG-RAN node’s cell re-activation reply from the NG-RAN node over the S1 interface and NG interface. Upon reception of the re-activation request, the NG-RAN node’s cell should remain switched on at least until expiration of the minimum activation time. The minimum activation time may be configured by O&M or be left to the NG-RAN node’s implementation.

Inter-system load balancing

The load reporting function for inter-system load balancing is executed by exchanging load information between NG-RAN and E-UTRAN. Both event-triggered and periodic inter-system load reporting are supported. Event-triggered inter-system load reports are sent when the reporting node detects crossing of cell load thresholds.

The following load related information should be supported:

-	Cell Capacity Class value (UL/DL relative capacity indicator);

-	Capacity value (per cell: UL/DL available capacity);

-	RRC connections (number of RRC connections, and available RRC Connection Capacity);

-	Number of active UEs.

-	PRB usage (per cell: UL/DL)

NGAP procedures used for inter-system load balancing are Uplink RAN Configuration Transfer and Downlink RAN Configuration Transfer.

S1AP procedures used for inter-system load balancing are eNB Configuration Transfer and MME Configuration Transfer.

2-step RACH optimization

2-step RACH optimization is supported by UE reported 2-step RACH related information made available at the NG RAN node and by PRACH parameters exchange between NG RAN nodes.

PCI selection

For aggregated architecture and centralized PCI assignment in gNB, the OAM assigns a single PCI for each NR cell in the gNB, and the gNB selects this value as the PCI of the NR cell.

For Aggregated architecture and distributed PCI assignment in gNB, the OAM assigns a list of PCIs for each NR cell in the gNB, and the gNB selects a PCI value from the list of PCIs. The gNB may restrict this list by removing some PCIs that are reported by UEs, reported over the Xn interface by neighboring gNBs, and/or acquired through other methods, e.g. detected over the air using a downlink receiver.

The PCI Optimization Function in split gNB case, the OAM configures a PCI for each NR cell to the gNB-DU. For centralized PCI assignment in split gNB architecture, the gNB-CU detects PCI conflict of NR cells and reports the NR cells suffering PCI confilict to OAM directly. The OAM is in charge of reassigning a new PCI for the NR cell subject to PCI conflict. For distributed PCI assignment in split gNB architecture, the OAM assigns a list of PCIs for each NR cell and sends the configured PCI list to the gNB-CU. If the gNB-CU detects PCI conflict, the gNB-CU may select a new PCI value from the preconfigured PCI list for the NR cell and send it to the gNB-DU by either F1 Setup procedure or gNB-CU configuration update procedure.

Energy efficiency (OAM requirements)

To calculate the energy efficiency of base stations, ETSI ES 203 228 ("Environmental Engineering (EE); Assessment of mobile network energy efficiency") defines the following high-level EE KPI:

In which Mobile Network data Energy Efficiency (EEMN,DV) is the ratio between the performance indicator (i.e. Data Volume DVMN) and the energy consumption (ECMN).

Successful Handovers Reports

Successful Handovers Reports is reported by the UE to detect failure events happened during successful handovers.

The solution for the problem may consist of the following steps:

1)	UE is configured with triggering conditions to compile the Successful Handover Report;

2)	Only if the conditions are met, UE generates Successful Handover Report comprising a set of measurements collected during the successful handover phase, i.e. measurement at the handover trigger, measurement at the end of handover execution or measurement after handover execution.

3) The availability of a Successful Handover Report may be indicated by Completed message (RRCReconfigurationComplete, RRCReestablishmentComplete, RRCSetupComplete, RRCResumeComplete) transmitted from UE to NG-RAN node over RRC. The NG-RAN node may fetch information of a successful handover report via UE Information Request/Response mechanism.

4) NG-RAN node could forward the Successful Handover Report to the source NR-RAN node to indicate failures experienced during a successful handover event.

Upon reception of a Successful HO Report, the receiving node is able to analyse whether its mobility configuration needs adjustment.

UE history information in EN-DC

UE history information is introduced in EN-DC to avoid Ping Pong effect. The MN stores and correlates the UE History Information from MN and SN(s) as long as the UE stays in MR-DC, forwards UE History Information and optional UE History Information from the UE to its connected SNs. The resulting information is then used by SN in subsequent handover preparation. The SN is in charge of collecting SCG UE history information and providing the collected information to the MN based on MN request or MN subscription on the PSCell change.

The MN may retrieve the SCG UE history information via the SN Addition and SN Modification procedures. SN shall provide the SCG UE history information, if available, in the SN Addition, SN Modification, SN Release, and SN initiated SN Change procedures.

Load balancing enhancement

The load reporting function is executed by exchanging load information over the Xn/X2/F1/E1 interfaces. Besides the load metrics introduced in Rel-16, some more metrics are introduced for intra-system load balancing, including, PRB usage for slice(s): DL/UL GBR PRB usage, DL/UL non-GBR PRB usage, and DL/UL Total PRB allocation) and PRB utilisation for MIMO

To achieve load reporting function, Resource Status Reporting Initiation & Resource Status Reporting procedures are used.

MRO for SN change failure

For analysis of PSCell change failure, the UE makes the SCG Failure Information available to the MN.

MN performs initial analysis to identify the node that caused the failure. If the failure is caused by a SN, the MN forwards the SCG Failure Information to the SN. The SN performs the final root cause analysis. The details of the solution, including the description in this paragraph are FFS.

One of the functions of self-optimization for PSCell change is to detect PSCell change failures that occur due to Too late PSCell change or Too early PSCell change, or Triggering PSCell change to wrong PSCell. These problems are defined as follows:

-	Too late PSCell change: an SCG failure occurs after the UE has stayed for a long period of time in the PSCell; a suitable different PSCell is found based on the measurements reported from the UE.

-	Too early PSCell change: an SCG failure occurs shortly after a successful PSCell change from a source PSCell to a target PSCell or a PSCell change failure occurs during the PSCell change procedure; source PSCell is still the suitable PSCell based on the measurements reported from the UE.

-	Triggering PSCell change to wrong PSCell: an SCG failure occurs shortly after a successful PSCell change from a source PSCell to a target PSCell or a PSCell change failure occurs during the PSCell change procedure; a suitable PSCell different with source PSCell or target PSCell is found based on the measurements reported from the UE.

MDT enhancement

-	Support for NR MDT IDC mechanism

-	Extension to LoggedMeasurementConfiguration with a flag to indicate if an early measurement/idle mode configuration has relevance for logged measurement purposes

-	UE support to assist the network in preventing management based logged MDT overwriting signalling based logged MDT

-	Extension to LoggedMeasurementConfiguration with Logged MDT type indication used for signalling MDT protection

-	RACH failure report extension with 2-step RACH relevant information

-	Support for multiple CEF reports

-	Support of M5~M7 for EN-DC SN terminated MCG bearer/split bearers and MN terminated SCG/split bearers

-	Immediate MDT configuration support for MN terminated SCG bearer and SN terminated MCG/split bearer by the the terminated node, e.g., MN in case of MN terminated SCG bearer

-	RLF report support for CHO and DAPS HO

-	Support for logging of on-demand SI

L2 measurement

PRB usage for MIMO was first introduced to NR in Rel-16 to reflect the PRB usage at the case of MU-MIMO and multiple MIMO layers. Configuring the same constant value Alpha for all the cells sometimes is not suitable, especially for cells in bad radio condition. And it is also difficult to manually configure Alpha for each cell, considering the large number of NR base stations. In Rel-17, PRB Usage based on statistical MIMO layer and Enhanced PRB Usage for MIMO are specified. Comparing with R16 PRB usage measurement, the new PRB usage measurement can adjust Alpha autonomously, e.g., based on statistical data of MIMO layer. The objectives of the measurements are to measure usage of time and frequency resources. A use-case is OAM performance observability.

In addition, PDCP excess packet delay is also specified for delay sensitive services, e.g., URLLC. The objective of this measurement performed by UE is to measure Excess Packet Delay in Layer PDCP for QoS verification of MDT.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=860053,860153

[1]	RP-22xxxx, Status report for WI on enhancement of SON_MDT support for NR and MR-DC, CMCC

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 910029 | Plug and connect support for management of Network Functions | PACMAN | S5 | SP-210260 | Per Elmdahl, Ericsson |
| 870028 | Enhancements of Self-Organizing Networks (SON) for 5G networks | eSON_5G | S5 | SP-200194 | Joey Chou, Intel |
| 850032 | Study on autonomous network levels | FS_ANL | S5 | SP-190928 | China Mobile |
| 880027 | Autonomous network levels | ANL | S5 | SP-200464 | Cao, Xi, China Mobile |
| 900031 | Enablers for Network Automation for 5G - phase 2 | eNA_Ph2 |  | SP-200975 | TBD |
| 840022 | Study on Enablers for Network Automation for 5G - phase 2 | FS_eNA_Ph2 | S2 | SP-200098 | Xiaobo Wu, Huawei Technologies |
| 890015 | Study on security aspects of enablers for Network Automation (eNA) for 5GS Phase 2 | FS_eNA_SEC | S3 | SP-200722 | Xiaoting Huang, China Mobile |
| 900010 | Stage 2 of eNA_Ph2 | eNA_Ph2 | S2 | SP-200975 | TBD |
| 910012 | CT aspects of eNA_Ph2 | eNA_Ph2 | ct | CP-211191 | Huang Zhenning (China Mobile) |
| 910088 | CT3 aspects of eNA_Ph2 | eNA_Ph2 | C3 | CP-211191 | Huang Zhenning (China Mobile) |
| 910089 | CT4 aspects of eNA_Ph2 | eNA_Ph2 | C4 | CP-211191 | Huang Zhenning (China Mobile) |
| 930007 | Security aspects of eNA_Ph2 | eNA_Ph2 | S3 | SP-210837 | Chang Liu, China Mobile |

Summary based on the input provided by XXX in xP-22xxxx.

## 8.5	Minimization of service Interruption

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 850045 | Support for Minimization of service Interruption | MINT |  | SP-190814 | SungDuck Chun; LG Electronics |
| 830018 | Study on MINT | FS_MINT | S1 | SP-190090 | SungDuck Chun; LG Electronics |
| 850036 | Stage 1 of MINT | MINT | S1 | SP-190938 | SungDuck Chun; LG Electronics |
| 920062 | Stage 2 of MINT | MINT | S2 | SP-210582 | Hyunsook Kim, LG Electronics |
| 900004 | Study on the CT aspects of Support for Minimization of service Interruption | FS_MINT-CT | C1 | CP-203232 | Sang Min Park, LG Electronics |
| 930003 | Stage 3 of MINT | MINT | ct | CP-212166 | Sang Min Park |
| 930049 | CT1 aspects of MINT | MINT | C1 | CP-212166 | Sang Min Park |
| 930045 | CT4 aspects of MINT | MINT | C4 | CP-212166 | Sang Min Park |
| 930048 | CT6 aspects of MINT | MINT | C6 | CP-212166 | Sang Min Park |

Summary based on the input provided by XXX in xP-22xxxx.

# 9	Multicast-Broadcast Services (MBS) and Mission Critical (MC)

| 890039 | Mission Critical Data | eMCData3 |  | SP-191106 | Shih, Jerry, AT&T |
| --- | --- | --- | --- | --- | --- |
| 860007 | Stage 2 of MCData3 | eMCData3 | S6 | SP-191106 | Shih, Jerry, AT&T |
| 890038 | CT aspects of eMCData3 | eMCData3 | C1 | CP-201177 |  |

Summary based on the input provided by at&t in SP-220089.

For Release 17, the enhancements for the MCData service were defined by the two work items above. The following functionalities have been introduced:

- new network-based MCData notification server. The MCData notification server provides the centralized notification function in the network that allows an application (e.g. resident in the UE) to create a communication channel to receive real-time notifications from the network in either Pull or Push mode.

- MCData communication (SDS or file distribution) supports using functional alias as target end points, except FD using HTTP.

- new "search folder" and "retrieve folder content" operations to MCData message store operations.

- Support application specific metadata container in MCData communication for application specific handling.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=890039,860007,890038

[1]	TS 22.282 Mission Critical Data services; Stage 1;

[2]	TS 22.280 Mission Critical Services Common Requirements (MCCoRe); Stage 1;

[3]	TS 23.282 Functional architecture and information flows to support Mission Critical Data (MCData) Stage 2 (together with 23.280, it specifies the architecture, including information flows, procedures, and configuration) ;

[4]	TS 23.280 Common functional architecture to support mission critical services; Stage 2;

[5]	TS 23.303 Proximity-based services (ProSe); Stage 2 (ProSe is an enabler for MC services);

[6]	TS 23.468 Group Communication System Enablers for LTE (GCSE_LTE); Stage 2;

[7]	TS 24.282 Mission Critical Data (MCData) signalling control (specifies the protocol aspects for call control);

[8]	TS 24.582 Mission Critical Data (MCData) media plane control(specifies the protocol aspects for media plane);

[9]	TS 24.481 Mission Critical Services (MCS) group management; Protocol specification;

[10]	TS 24.482 Mission Critical Services (MCS) identity management; Protocol specification;

[11]	TS 24.483 Mission Critical Services (MCS) Management Object (MO);

[12]	TS 24.484 Mission Critical Services (MCS) configuration management; Protocol specification;

[13]	TS 29.213 Policy and Charging Control signalling flows and Quality of Service (QoS) parameter mapping;

[14]	TS 29.214: Policy and Charging Control over Rx reference point;

[15]	TS 29.283: Diameter data management application (specifies the protocol aspects for data management related to MC service user profile )

[16]	TS 33.180: Security of the mission critical service.

| 890003 | CT aspects of Enhanced Mission Critical Communication Interworking with Land Mobile Radio Systems | eMCCI_CT | C1 | eMCCI_CT | Mike Dolan, FirstNet |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by FirstNet in CP-220110.

eMCCI_CT refers to stage 3 aspects of the stage 2 work defined by eMCCI in Rel-16.

Note: 	This does not comply with the 3GPP methodology: all stages shall be defined within a same Release and use the same Feature/acronym.

It covers enhancements to interworking 3GPP mission critical systems with Land Mobile Radio (LMR, i.e. public safety communication networks) systems in Rel-17 to provide support for a conference event package, affiliation on behalf of a set of users, and private call floor control.

The Rel-17 IWF (interworking function) now supports the ability for the IWF to handle subscriptions to events that may occur in the LMR system, and for the IWF to be able to subscribe on behalf of LMR users to events in the 3GPP mission critical system (MCPTT and MCData).

Affiliation on behalf of a set of users is now handled from the IWF toward the 3GPP mission critical system to potentially reduce the signalling load due to repetitive messaging.

Private call floor control is now supported between an MCPTT system and an IWF.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=890003

[1]	TS 29.379: “Mission Critical Push To Talk (MCPTT) call control interworking with Land Mobile Radio (LMR) systems”

[2]	TS 29.380: “Mission Critical Push To Talk (MCPTT) media plane control interworking with Land Mobile Radio (LMR) systems”

[3]	TS 29.582: “Mission Critical Data (MCData) interworking with Land Mobile Radio (LMR) systems”

| 870034 | Enhanced Mission Critical Push-to-talk architecture phase 3 | enh3MCPTT |  | SP-200108 | Dom Lazara; Motorola Solutions |
| --- | --- | --- | --- | --- | --- |
| 870016 | Stage 2 aspects of enh3MCPTT | enh3MCPTT | S6 | SP-200108 | Dom Lazara; Motorola Solutions |
| 900039 | Stage 3 aspects of enh3MCPTT | enh3MCPTT-CT | C1 | CP-202188 |  |

Summary based on the input provided by FirstNet in CP-220111.

This WI covers work on enhancements to the MCPTT feature as to provide support for an MCPTT user logged into multiple devices, support for z-axis location measurements, preconfigured regroup support, and unicast media stop/resume.

The Rel-17 system now supports multiple registrations for the same mission critical user, such as might happen if a public safety person uses a tablet, an in-vehicle computer, and a smartphone.

Location services reports can now include z-axis information in support of situational awareness, such as might be needed to determine what floor of a building a firefighter is on.

User and group regroups can now be based on a group that is to be used only for preconfigured regroups. Clients in the device will reject attempts to make group calls directly to groups marked for preconfigured group use only. In addition, the preconfigured group capabilities have been added to MCVideo and MCData.

For MCPTT, it is now possible for a client to indicate to the server that it wants to stop/resume receiving media for a given group call. This may be needed to preserve radio resources when the MCPTT user is participating in another higher priority group call.

Finally, a number of issues that were found during ETSI Plugtest events have been fixed or clarified.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=870034,870016,900039

[1]	TS 24.281: “Mission Critical Video (MCVideo) signalling control”

[2]	TS 24.282: “Mission Critical Data (MCData) signalling control”

[3]	TS 24.379: “Mission Critical Push To Talk (MCPTT) call control”

[4]	TS 24.380: “Mission Critical Push To Talk (MCPTT) media plane control”

[5]	TS 24.483: “Mission Critical Services (MCS) Management Object (MO)”

[6]	TS 24.484: “Mission Critical Services (MCS) configuration management”

| 890011 | Mission critical security enhancements phase 2 | MCXSec2 | S3 | SP-200879 | Woodward, Tim, Motorola Solutions, Inc. |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Motorola Solutions in SP-220019.

Mission critical (MC) services security enhancements phase 2 defines the confidentiality, integrity, user authentication, service authorization, and overall security architecture for Release 17 mission critical services (MCPTT, MCVideo, MCData, MC Location, MC Interworking, MC Interconnection, and MC Railway).

Release 17 expands on the mission critical security architecture already defined in previous releases and includes some mission critical security clarifications and corrections.

In this release, mission critical user service authorization and security for the mission critical MCData message store service.  Similar to user service authorization for the other MC services, an appropriately scoped access token obtained from the Identity Management server permits only authorized users the authorization to access and use the MCData message store service.

Security for Preconfigured Group Regroup and Preconfigured User Regroup calls defines the use of the preconfigured group to establish the security context.

Enhancements to the security architecture to support mission critical security services over a 5G system.  This includes the mission critical security architecture, which describes the use and integration of 5G nodes and servers.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=890011

[1]	TS 33.180: “Security of the Mission Critical (MC) service; (Release 17)”

| 860048 | NR multicast and broadcast services | NR_MBS | R2 | RP-220428 | Huawei |
| --- | --- | --- | --- | --- | --- |
| 860148 | Core part: NR multicast and broadcast services | NR_MBS-Core | R2 | RP-220428 | Huawei |

Summary based on the input provided by Huawei, HiSilicon in RP-220408.

This Rel-17 NR MBS WI specifies two delivery modes, i.e. MBS multicast and MBS broadcast, for services of PTM (Point-To-Multipoint) nature, such as for example public safety and mission critical services, V2X applications, IPTV, live video, software delivery over wireless and IoT applications.

Before introducing NR MBS, there was no broadcast/multicast transmission supported in NR for user data delivery. Services of PTM nature could only be delivered over NR based on unicast, which is inefficient, in particular from radio resources utilization point of view. Nevertheless, for the use cases mentioned above, broadcast/multicast transmission provides substantial benefits, especially in terms of system efficiency and user experience. The MBS multicast delivery mode is capable of addressing higher QoS services while the MBS broadcast delivery mode is focusing on lower QoS services.

The objectives of NR MBS WI are included in [2], and the WI is linked to the SA2 WI on Architectural enhancements for 5G multicast-broadcast services [3].

MBS Multicast

MBS multicast provides the MBS delivery mode for RRC_CONNECTED mode UEs, with the following characteristics:

Group scheduling

A common frequency resource (CFR) is defined for multicast scheduling as an ‘MBS frequency region’ with a number of contiguous PRBs, which is configured within the dedicated unicast BWP. A group of UEs can be configured via RRC signalling with a G-RNTI for group scheduling, and the group of UEs can also be configured with downlink SPS and G-CS-RNTI for MBS multicast. The gNB schedules a transport block using G-RNTI (or G-CS-RNTI) to the group of UEs.

HARQ feedback

HARQ feedback is used to further improve the group scheduling efficiency, and the following two HARQ feedback reporting modes are supported:

	In the first HARQ feedback reporting mode, the UE transmits a PUCCH with HARQ-ACK information if the UE has correctly received the transport block or HARQ-NACK value if the UE has not correctly received the transport block.

	For the second HARQ feedback reporting mode, the UE transmits a PUCCH with HARQ-NACK information only if the UE has not correctly received the transport block.

HARQ reporting for multicast can also be disabled for a UE either semi-statically or dynamically.

Dynamic PTP (Point-To-Point)/PTM switch for MBS multicast

It is not always efficient for a gNB to schedule data based on G-RNTI (PTM), and sometimes PTP based scheduling (same as unicast) can bring more benefits thanks to the advanced unicast mechanisms. Based on the common PDCP entity, the gNB can decide whether to use PTM or PTP to deliver data of an MBS multicast session to the UE(s) at a certain time. The gNB makes its decision based on information such as MBS Session QoS requirements, the number of jointly scheduled UEs, UE feedback on link quality, and other criteria and ensures QoS requirements to be met for the service regardless of the chosen transmission method.

Figure 1: Dynamic PTP/PTM switch for MBS multicast

Lossless handover for MBS multicast

To support high QoS services, it is necessary to ensure lossless data delivery also during a handover. To enable lossless handover, synchronisation of PDCP SNs among source and target RAN nodes should be ensured, by either or a combination of the following methods:

-	Derivation of the PDCP SNs from DL MBS QFI SNs provided on NG-U;

-	Deployment of a Shared NG-U Termination at NG-RAN, shared among gNBs, which comprises a common entity for assignment of PDCP SNs.

Figure 2: PDCP SN synchronization to enable lossless

MBS Broadcast

MBS broadcast provides the downlink only MBS delivery mode for UE in all RRC states, addressing lower QoS services, with the following characteristics:

Group scheduling

A common frequency resource (CFR) is defined for broadcast scheduling as an ‘MBS frequency region’ with a number of contiguous PRBs in which G-RNTI can be used to schedule the associated MBS broadcast service. The bandwidth of CFR can be equal to or larger than initial BWP, which is indicated by system information. HARQ feedback and HARQ re-transmission is not supported for MBS broadcast.

MBS broadcast Configuration

The UE can receive the MBS configuration for a broadcast service via a broadcast control channel, i.e. MCCH, which is the same for UEs in RRC_IDLE , RRC_INACTIVE and RRC_CONNECTED states.

Service continuity

Lossless mobility cannot be ensured for MBS broadcast, but some mechanisms are specified to support service continuity of the broadcast service. NR MBS broadcast supports MBS frequency prioritization, which enables the UE in RRC_IDLE/RRC_INACTIVE to select the right frequency to camp on and receive its services of interest.

To ensure service continuity of MBS broadcast for UEs in RRC_CONNECTED, the UE can send MBS Interest Indication to the gNB and the gNB can configure the UE in a way allowing it to receive the services the UE is interested in using MBS broadcast.

References

List of related CRs: select "TSG Status = Approved" in:

https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=

[1]	RP-220407, Status report for WI NR Multicast and Broadcast Services, Huawei, HiSilicon;

[2] 	TS 33.180: “Security of the Mission Critical (MC) service; (Release 17)”

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 900038 | Multicast-broadcast services in 5G | 5MBS |  | SP-201106 | Meng Li, Huawei |
| 830030 | Study on Architectural enhancements for 5G multicast-broadcast services | FS_5MBS | S2 | SP-200690 | Meng Li, Huawei |
| 900009 | Stage 2 for 5MBS | 5MBS | S2 | SP-201106 | Meng Li, Huawei |
| 920043 | CT1 aspects of 5MBS | 5MBS | C1 | CP-212022 | Gulbani, Giorgi, Huawei |
| 920044 | CT3 aspects of 5MBS | 5MBS | C3 | CP-212022 | Gulbani, Giorgi, Huawei |
| 910002 | CT4 aspects of 5MBS | 5MBS | C4 | CP-212022 | Gulbani, Giorgi, Huawei |
| 920023 | Security Aspects of Enhancements for 5G Multicast-Broadcast Services | 5MBS | S3 | SP-210420 | Longhua Guo, Huawei Technologies |
| 870014 | Study on Multicast Architecture Enhancements for 5G Media Streaming | FS_5GMS_Multicast | S4 | SP-200238 | Peng Tan, Telus |
| 810055 | Study on location enhancements for mission critical services | FS_enhMCLoc | S6 | SP-190725 | Dom Lazara, Motorola Solutions |
| 850040 | Broadcast / Multicast requirements supporting Mission Critical Services in 5G | 5MBS_eMC | S1 | SP-190942 | Toobe, Jens, BDBOS |
| 880006 | Study on Security Aspects of Enhancements for 5G Multicast-Broadcast Services | FS_5MBS_SEC | S3 | SP-200351 | Longhua Guo, Huawei Technologies |
| 850035 | Study on Mission Critical services over 5G multicast-broadcast system | FS_MC5MBS | S6 | SP-190929 | Val Oprescu, AT&T |
| 800023 | Study on Mission Critical services support over 5G System | FS_MCOver5GS | S6 | SP-200837 | Verweij, Kees, The Police of the Netherlands |
| 920051 | Mission Critical Services over 5GS | MCOver5GS |  | SP-200833 | Wendler, Ingo, UIC |
| 890027 | Stage 2 of MCOver5GS | MCOver5GS | S6 | SP-200833 | Wendler, Ingo, UIC |
| 920007 | CT1 aspects of MCOver5GS | MCOver5GS | C1 | CP-211118 | Gkatzikis, Lazaros, Nokia |
| 820040 | Mission Critical system migration and interconnection | MCSMI_CT | C1 | CP-190143 | Dom Lazara, Motorola Solutions |

Summary based on the input provided by XXX in xP-22xxxx.

# 10	Multi-(U)SIM

| 860063 | Support for Multi-SIM devices for LTE/NR | LTE_NR_MUSIM |  | RP-212610 | Vivo |
| --- | --- | --- | --- | --- | --- |
| 860163 | Core part: Support for Multi-SIM devices for LTE/NR | LTE_NR_MUSIM-Core | R2 | RP-212610 | Vivo |

Summary based on the input provided by vivo in RP -220604.

This work item specifies solutions to address the paging occasion collision issue for single-Rx/single-Tx Multi-USIM (MUSIM) UE, and to notify network A of its switch from network A for MUSIM purpose, and to support indicating to the MUSIM UE whether an incoming paging is for voice service.

The following schemes were introduced as part of the Work Item:

-	Paging collision objective: To solve paging occasion collision problem, the MUSIM UE can trigger a new 5G-GUTI reallocation in 5GS or an IMSI offset assignment in EPS to modify the timing of the paging occasions. In 5GS, the UE obtains a new 5G-GUTI by performing a MRU without any specific indication. In EPS, the UE can provide a requested IMSI offset value in Attach Request or TAU Request, which triggers the MME to provide an accepted IMSI Offset value in the Attach Accept or TAU Accept message. The MME and UE use the alternative IMSI (calculated based on the IMSI and the accepted IMSI offset) for the determination of paging occasion.

-	Network notification objective: AS-based network switching for leaving RRC_CONNECTED and network switching without leaving RRC_CONNECTED (i.e., requesting/configuring MUSIM gaps) were introduced. Both schemes can be configured by the network separately.

When determining it needs to leave RRC_CONNECTED, the UE sends the UEAssistanceInformation message, which indicates the UE’s preferred RRC state when leaving RRC_CONNECTED for MUSIM purpose. gNB may release the UE to RRC IDLE/INACTIVE when receiving this UAI. The UE is allowed to enter RRC_IDLE if it does not receive response message from network within a certain configured time.

When determining it needs the MUSIM gaps, the UE sends the UEAssistanceInformation message, which indicates the UE’s preference on the MUSIM gaps. The UE can request at most a single aperiodic MUSIM gap and two periodic MUSIM gaps. The MUSIM gap is per UE level.

-	Paging cause objective: a paging cause field with only one codepoint voice was introduced in the NR/LTE Uu paging message, S1AP/NGAP paging message, and XNAP/F1AP paging message. The network provides this field for the upcoming paging triggered by IMS voice, only if the UE indicates the paging cause feature is supported to the network. In order to enable NG-RAN to deliver the paging cause in RAN paging for the UE in RRC-INACTIVE, the AMF provides an indication indicating the Paging Cause Indication for Voice Service feature is supported to the NG-RAN. The NG-RAN node knows the downlink data which triggers the RAN Paging message is related to voice service based on the Paging Policy Indicator, in the header of the received downlink data.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=860063,860163

[1]	RP-220603, Status report for WI

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 840049 | Support for Multi-USIM Devices | MUSIM |  | SP-190309 | Liao, Ellen C, Intel |
| 830019 | Study on MUSIM | FS_MUSIM | S1 | SP-190091 | Liao, Ellen C, Intel |
| 840040 | Stage 1 of MUSIM | MUSIM | S1 | SP-190309 | Liao, Ellen C, Intel |
| 820012 | Study on Stage 2 (System Enablers) for MUSIM | FS_MUSIM | S2 | SP-200297 | Sašo Stojanovski, Intel |
| 900013 | System enablers for Multi-USIM devices | MUSIM | S2 | SP-210091 | Sašo Stojanovski, Intel |
| 910015 | CT aspects of MUSIM | MUSIM | ct | CP-212102 | Vivek Gupta, Intel |
| 910063 | CT1 aspects of Enabling Multi-USIM Devices | MUSIM | C1 | CP-212102 | Vivek Gupta, Intel |
| 910064 | CT4 aspects of Enabling Multi-USIM Devices | MUSIM | C4 | CP-212102 | Vivek Gupta, Intel |
| 900017 | Study on the security of the system enablers for devices having multiple Universal Subscriber Identity Modules (USIM) | FS_MUSIM_SEC | S3 | SP-201018 | Abhijeet Kolekar, Intel Corporation |

# 11	Messaging improvements

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 930030 | Support of the 5GMSG Service | FS_5GMARCH | S6 | SP-200835 | Liu, Yue, China Mobile |
| 840036 | Study on support of the 5GMSG Service | FS_5GMARCH | S6 | SP-200835 | Liu, Yue, China Mobile |
| 890010 | Study on security aspects of the 5GMSG Service (St1 in Rel16) | FS_SEC_5GMSG | S3 | SP-200878 | Xiaoting Huang, China Mobile |
| 930005 | Security aspects of the 5GMSG Service | 5GMSG | S3 | SP-210835 | Xiaoting Huang, China Mobile |
| 930040 | Application Architecture for MSGin5G Service | 5GMARCH |  | SP-200832 | Liu, Yue, China Mobile |
| 890026 | Application Architecture for MSGin5G Service | 5GMARCH | S6 | SP-200832 | Liu, Yue, China Mobile |
| 930004 | CT aspects for enabling MSGin5G Service | 5GMARCH | ct | CP-212106 | Liu, Yue, China Mobile |
| 930041 | CT1 aspects for enabling MSGin5G Service | 5GMARCH | C1 | CP-212106 | Liu, Yue, China Mobile |
| 930042 | CT3 aspects for enabling MSGin5G Service | 5GMARCH | C3 | CP-212106 | Liu, Yue, China Mobile |

Summary based on the input provided by China Mobile in SP-220285.

This Feature improves the messaging communication capability of the 5G System, especially for Massive Internet of Things (MIoT). It is based on the study in TR 23.700-24 [4].

Note: 	The different stages have been spread on different Releases, and on different Features, using different acronyms: Stage 1 is defined in Rel-16 by "MSGin5G" (TS 22.262 [2]), Stages 2 (TS 23.554 [1] ) and Stage 3 are defined in Rel-17 by "5GMARCH" and Security aspects have been defined in Rel-17 by "5GMSG". This does not comply with the 3GPP methodology (all stages shall be defined within a same Release and using the same Feature/acronym). The name used to refer to this service in the specifications and in this summary is "MSGin5G".

The following figure, from TS 23.554 [1], provides the high level illustration of the MSGin5G service:

Figure 1: Application Architecture of the MSGin5G Service

The following functions are specified:

a)	Configuration of MSGin5G UE and Non-MSGin5G UE to get the MSGin5G Service configuration information (e.g. UE Service ID);

b)	Registration of MSGin5G UE and registration of Message Gateway on behalf of the Non-3GPP UEs;

c)	Message delivery procedures for the message communication models listed below (the message can be delivered between different PLMNs):

•	Point-to-point message which happens between a person and a thing or two things;

•	Application-to-point message/ Point-to-application message which happens between an application server and an IoT device;

•	Group message which originates at a UE and terminated at a group of UEs, all members in the group can send and receive the message;

•	Broadcast message which originates at an application sever in the network or an UE and terminated at all the UEs in a specific service area within a cell or multiple cells;

•	Message delivery based on messaging topic which the message is delivered to all subscribers (UE or Application Server) of this messaging topic.

d)	Message Aggregation can be used to optimize communications towards the same target by aggregating one or more messages into a single message;

e)	MSGin5G message can be segmented transmission if the content is larger than the maximum payload length of a message and reassembled by the suitable recipient;

f)	MSGin5G message can be stored by the MSGin5G Server if a UE is unavailable (disconnected or power off) for future delivery once the UE becomes available;

g)	3GPP network functionalities of UE reachability status monitoring and device triggering can be leveraged by the MSGin5G Service via Core Network exposure.

h)	The Service Enabler Architecture Layer for Verticals (SEAL) capabilities "Group management service" and "Configuration management service" can be used by the MSGin5G service. These services are specified in TS 23.434 [6] and TS 23.554 [1].

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=930030,840036,890010,930005,930040,890026,930004,930041,930042

[1]	TS 23.554: "Application architecture for MSGin5G Service; Stage 2" (specifies functional architecture, procedures, information flows and APIs )

[2]	TS 22.262: "Message Service within the 5G System".

[3]	TS 33.501: "Security architecture and procedures for 5G System".

[4]	TS 24.538: "Enabling MSGin5G Service;Protocol specification" (specifies detailed procedures over CoAP protocol between MSGin5G Client and MSGin5G Server)

[5]	TS 29.538: "Enabling MSGin5G Service;Application Programming Interfaces (API) specification;" (specifies the RESTful APIs provided by MSGin5G Server towards the Application Server and Message Gateway)

[6]	TS 23.434: "Service Enabler Architecture Layer for Verticals".

| 890001 | Service-based support for SMS in 5GC | SMS_SBI |  | CP-212023 | HAOUARI, Wafa, Orange |
| --- | --- | --- | --- | --- | --- |
| 890028 | CT1 aspects of SMS_SBI | SMS_SBI | C1 | CP-212023 | HAOUARI, Wafa, Orange |
| 890029 | CT4 aspects of SMS_SBI | SMS_SBI | C4 | CP-212023 | HAOUARI, Wafa, Orange |

# 12	Drone/UAS/UAV/EAV identification

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 840083 | 5G Enhancement for UAVs | EAV |  | SP-190308 | Qun Wei, China Unicom |
| 810019 | Study on EAV | FS_EAV | S1 | SP-180909 | Qun Wei, China Unicom |
| 840039 | Stage 1 of EAV | EAV | S1 | SP-190308 | Qun Wei, China Unicom |
| 820011 | Study on supporting Uncrewed Aerial Systems Connectivity, Identification, and Tracking | FS_ID_UAS_SA2 | S2 | SP-200097 | Qualcomm, Stefano Faccin |
| 820026 | Study on application layer support for Uncrewed Aerial System (UAS) | FS_UASAPP | S6 | SP-200111 | Niranth Amogh, Huawei |
| 920045 | Application layer support for Uncrewed Aerial System (UAS) | UASAPP |  | SP-200988 | Niranth Amogh, Huawei Telecommunications India |
| 900025 | Stage 2 of Application layer support for Uncrewed Aerial System (UAS) | UASAPP | S6 | SP-200988 | Niranth Amogh, Huawei Telecommunications India |
| 920004 | CT1 Aspects of Application Layer Support for Uncrewed Aerial Systems (UAS) | UASAPP | C1 | CP-211111 | Lin Shu, Huawei, |
| 920046 | CT3 Aspects of Application Layer Support for Uncrewed Aerial Systems (UAS) | UASAPP | C3 | CP-211111 | Lin Shu, Huawei |
| 880007 | Study on security aspects of Uncrewed Aerial Systems (UAS) | FS_UAS_SEC | S3 | SP-200352 | Adrian Escott, Qualcomm |
| 810049 | Remote Identification of Uncrewed Aerial Systems | ID_UAS |  | SP-180771 | Eddy HALL, Qualcomm |
| 810013 | Stage 1 of Remote Identification of Uncrewed Aerial Systems | ID_UAS | S1 | SP-180771 | Eddy HALL, Qualcomm |
| 900014 | (Stage 2 of) Support of Uncrewed Aerial Systems Connectivity, Identification, and Tracking | ID_UAS | S2 | SP-200979 | Faccin, Stefano, Qualcomm |
| 910017 | CT aspects for ID_UAS | ID_UAS | ct | CP-211112 | Sunghoon Kim, Qualcomm |
| 910069 | CT1 aspects for ID_UAS | ID_UAS | C1 | CP-211112 | Sunghoon Kim, Qualcomm |
| 910070 | CT3 aspects for ID_UAS | ID_UAS | C3 | CP-211112 | Sunghoon Kim, Qualcomm |
| 910071 | CT4 aspects for ID_UAS | ID_UAS | C4 | CP-211112 | Sunghoon Kim, Qualcomm |
| 920028 | Security aspects of Uncrewed Aerial Systems | ID_UAS | S3 | SP-210425 | Escott, Adrian, Qualcomm |

Summary based on the input provided by XXX in xP-22xxxx.

# 13	Authentication and key management for applications based on 3GPP credential in 5G

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 890030 | Authentication and key management for applications based on 3GPP credential in 5G | AKMA |  | SP-190711 | Xiaoting Huang, China Mobile |
| 850021 | SA3 aspects of AKMA | AKMA | S3 | SP-190711 | Xiaoting Huang, China Mobile |
| 890008 | CT aspects of AKMA | AKMA-CT | ct | CP-203107 | Huang Zhenning (China Mobile) |
| 890031 | CT1 aspects of AKMA | AKMA-CT | C1 | CP-203107 | Huang Zhenning (China Mobile) |
| 890032 | CT3 aspects of AKMA | AKMA-CT | C3 | CP-203107 | Huang Zhenning (China Mobile) |
| 890033 | CT4 aspects of AKMA | AKMA-CT | C4 | CP-203107 | Huang Zhenning (China Mobile) |

Summary based on the input provided by China Mobile in SP-220289.

Authentication and key management for applications based on 3GPP credential in 5G (AKMA) is a cellular-network-based delegated authentication system specified for the 5G system, helping establish a secure tunnel between the end user and the application server. Using AKMA, a user can log in to an application service only based on the 3GPP credential which is the permanent key stored in the user’s tamper-resistant smart card UICC. The application service provider can also delegate the task of user authentication to the mobile network operator by using AKMA.

The AKMA architecture and procedures are specified by SA3 in TS 33.535 [1], with the related study showing how its general principles are derived documented in TR 33.835 [2]. The AKMA feature introduces a new Network Function into the 5G system, which is the AKMA Anchor Function (AAnF). Its detailed services and API definitions are specified by CT3 in TS 29.535[3]. Earlier generations of cellular networks include two similar standards specified by SA3, which are generic bootstrapping architecture (GBA) and battery-efficient security for very low throughput machine type communication devices (BEST). Since the AKMA feature is deemed as a successor of these systems, the work is launched by SA3 without the involvement of stage 1.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=890030,850021,890008,890031,890032,890033

[1]	TS 33.535: “Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)”

[2]	TR 33.835: “Study on authentication and key management for applications based on 3GPP credential in 5G”

[3]	TS 29.535: “5G System; AKMA Anchor Services; Stage 3”

# 14	Multimedia Priority Service (MPS) Phase 2

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 840046 | Multimedia Priority Service (MPS) Phase 2 | MPS2 |  | SP-190305 | Singh, Ray P; Perspecta Labs |
| 780005 | Study on MPS2 | FS_MPS2 | S1 | SP-190315 | Singh, Ray P; Vencore Labs |
| 850011 | Study on MPS2, Stage 2 | FS_MPS2_St2 | S2 | SP-190629 | Don Lukacs; Perspecta Labs |
| 840032 | Stage 1 of MPS2 | MPS2 | S1 | SP-190561 | Singh, Ray P; Perspecta Labs |
| 870002 | Stage 2 of MPS2 | MPS2 | S2 | SP-200519 | Streijl, Robert, Perspecta Labs |
| 880031 | Stage 3 of MPS2 | MPS2 | ct | CP-201207 | Peter Monnes, Perspecta Labs |
| 880050 | CT1 aspects of MPS2 | MPS2 | C1 | CP-201207 | Peter Monnes, Perspecta Labs |
| 880058 | CT3 aspects of MPS2 | MPS2 | C3 | CP-201207 | Peter Monnes, Perspecta Labs |
| 880051 | CT4 aspects of MPS2 | MPS2 | C4 | CP-201207 | Peter Monnes, Perspecta Labs |

Summary based on the input provided by XXX in xP-22xxxx.

# 15	NR enhancements

| 890056 | Introduction of DL 1024QAM for NR frequency range 1 (FR1) | NR_DL1024QAM_FR1 |  | RP-213654 | Ericsson |
| --- | --- | --- | --- | --- | --- |
| 890156 | Core part: Introduction of DL 1024QAM for NR FR1 | NR_DL1024QAM_FR1-Core | R4 | RP-213654 | Ericsson |
| 890256 | Perf. part: Introduction of DL 1024QAM for NR FR1 | NR_DL1024QAM_FR1-Perf | R4 | RP-213654 | Ericsson |

Summary based on the input provided by Ericsson in RP-220191.

This work item specifies downlink 1024QAM for NR PDSCH operation in FR1, which provides higher downlink peak rate compared with Release-15 NR, with the high downlink SINR and better channel condition (e.g., LOS or LOS-like channel), and with no mobility or very low mobility environment.

Beside specifying the downlink 1024QAM mapping function, the WI also defines the corresponding MCS/CQI tables and RRC signalling, as well as the corresponding BS/UE RF requirements to transmit/receive the signals with 1024QAM [2].

Introduction of the modulation mapping function of 1024QAM

It was introduced the modulation mapping function of 1024QAM for PDSCH, where 10 tuples of bits, {b_i,…,b_(i+9) } are mapped to complex-valued modulation symbols d(i), according to

di=16821-2bi16-1-2bi+28-1-2bi+44-1-2bi+62-1-2bi+8+j16821-2bi+116-1-2bi+38-1-2bi+54-1-2bi+72-1-2bi+9

Introduction of MCS table supporting 1024QAM

For supporting 1024QAM for PDSCH, a new five-bit MCS table with 1024QAM entries was introduced by removing 5 MCS entries and adding 5 new entries for 1024QAM from the existing MCS index table 2. The removed MCS indexes are 2, 4, 6, 8, and 10. The added MCS entries for 1024QAM are given as follows:

| MCS index | Modulation | Target code rate R x [1024] | Spectrum efficiency |
| --- | --- | --- | --- |
| 23 | 10 | 805.5 | 7.8662 |
| 24 | 10 | 853 | 8.3301 |
| 25 | 10 | 900.5 | 8.7939 |
| 26 | 10 | 948 | 9.2578 |
| 31 | 10 | reserved |  |

New RRC signalling was also introduced to indicate use of 1024QAM MCS table.

Introduction of CQI table supporting 1024QAM

For supporting 1024QAM for PDSCH, a new CQI table with 1024QAM entries was introduced by reusing LTE CQI table with 1024QAM entries as follows:

| CQI index | Modulation | Code rate x 1024 | Efficiency |
| --- | --- | --- | --- |
| 0 | Out of range |  |  |
| 1 | QPSK | 78 | 0.1523 |
| 2 | QPSK | 193 | 0.377 |
| 3 | QPSK | 449 | 0.877 |
| 4 | 16QAM | 378 | 1.4766 |
| 5 | 16QAM | 616 | 2.4063 |
| 6 | 64QAM | 567 | 3.3223 |
| 7 | 64QAM | 666 | 3.9023 |
| 8 | 64QAM | 772 | 4.5234 |
| 9 | 64QAM | 873 | 5.1152 |
| 10 | 256QAM | 711 | 5.5547 |
| 11 | 256QAM | 797 | 6.2266 |
| 12 | 256QAM | 885 | 6.9141 |
| 13 | 256QAM | 948 | 7.4063 |
| 14 | 1024QAM | 853 | 8.3301 |
| 15 | 1024QAM | 948 | 9.2578 |

New RRC signalling was also introduced to indicate use of 1024QAM CQI table.

BS Tx EVM for DL 1024QAM

The Error Vector Magnitude (EVM) is a measure of the difference between the ideal symbols and the measured symbols after the equalization. The required gNB Tx EVM for 1024QAM is set to 2.5% for frequencies equal to or below 4.2GHz and 2.8% for frequencies above 4.2GHz in FR1.

References

List of related CRs: select "TSG Status = Approved" in:

https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=890056,890156,890256

[1]	RP-220190	“Status report for WI Introduction of DL 1024QAM for NR FR1; rapporteur: Ericsson, Nokia”, Ericsson.

| 890058 | Enhanced NR support for high speed train scenario for frequency range 1 (FR1) | NR_HST_FR1_enh |  | RP-210833 | CMCC |
| --- | --- | --- | --- | --- | --- |
| 890158 | Core part: Enhanced NR support for high speed train scenario for FR1 | NR_HST_FR1_enh-Core | R4 | RP-210833 | CMCC |
| 890258 | Perf. part: Enhanced NR support for high speed train scenario for FR1 | NR_HST_FR1_enh-Perf | R4 | RP-210833 | CMCC |

Summary based on the input provided by CMCC in RP-220631.

This WI specifies the UE RRM requirements for carrier aggregation (CA) and UE demodulation requirements for CA for FR1 high speed train scenario (HST). The supported velocity is up to 500km/h and the carrier frequency is up to 3.6GHz covering both TDD and FDD. This WI is the enhancement of Rel-16 WI NR-HST (WID: RP-191512), which targets for single carrier scenario.

In Rel-17 WI on enhanced NR support for high speed train scenario for frequency range 1 (FR1), the enhanced RRM requirements and demodulation requirements for CA were specified to support the speed of up to 500km/h and carrier frequency of up to 3.6GHz.

For RRM, to guarantee the mobility performance for FR1 HST with velocity up to 500km/h，both enhanced requirements for NR inter-frequency measurement and enhanced requirements for CA scenario are specified in TS 38.133. The enhanced requirements for NR inter-frequency measurement include NR cell re-selection requirements, time period for PSS/SSS detection, time period for time index detection and measurement period requirements. The enhanced requirements for CA include both measurement on activated SCells and measurement on deactivated SCells.

For UE demodulation requirements for CA, both HST-SFN (High Speed Train Single Frequency Network) joint transmission scheme and DPS (Dynamic Point Selection) transmission scheme are considered, and the requirements are specified in TS38.101-4. With speed of up to 500km/h and carrier frequency of up to 3.6GHz, the maximum doppler shift is 1667Hz and 870Hz for 30KHz SCS and 15KHz SCS respectively. For 15KHz SCS, specify PDSCH requirements on single carrier of BW of {5, 10, 15, 20, 25, 30,35, 40, 45, 50} MHz. For 30KHz SCS, specify PDSCH requirements on single carrier of BW of {5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100} MHz.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=890058,890158,890258

| 890060 | NR support for high speed train scenario in frequency range 2 (FR2) | NR_HST_FR2 |  | RP-210800 | Samsung |
| --- | --- | --- | --- | --- | --- |
| 890160 | Core part: NR support for high speed train scenario in FR2 | NR_HST_FR2-Core | R4 | RP-210800 | Samsung |
| 890260 | Perf. Part: NR support for high speed train scenario in FR2 | NR_HST_FR2-Perf | R4 | RP-210800 | Samsung |

Summary based on the input provided by Ericsson in RP-220191.

This WI is targeted to specify NR UE RF requirements, RRM requirements and BS/UE performance requirements for train roof-mounted high-power devices in HST scenario with speed up to 350km/h and applicable frequency is up to 30GHz. The considered FR2 HST scenario is different from existing 3GPP WIs (for either LTE or NR) with operating bands up to 3.5GHz, and the NR FR2 RRM and demodulation requirements for non-HST scenarios are also not applicable and need to be specified.

In this WI, the FR2 HST deployment scenarios are studied, based upon which FR2 HST channel models are provided accordingly. From radio resource management (RRM) and demodulation perspectives, FR2 HST scenario is focused and evaluated, with the feasibility of FR2 HST scenario being technically confirmed.

For UE RF requirement, FR2 power class 6 for band n257, n258 and n261 is newly introduced, which is corresponding to the UE type of high speed train roof-mounted UE. Relevant UE TX and RX RF requirements for power class 6 are specified, in which the TX and EIS spherical coverage requirements are defined over the newly introduced spherical coverage evaluation areas. For UE beam correspondence requirement, the applicability rule and requirement side conditions are specified.

The RRM requirements for FR2 HST scenarios are introduced and enhanced over existing FR2 RRM requirements. Specifically, the enhancements for FR2 HST are introduced on cell re-selection, RRC connection mobility control, gradual timing adjustment, SSB based radio link monitoring and beam failure detection, intra-frequency measurement and L1-RSRP/SINR measurement. The new requirements of one shot large UL timing adjustment and TCI state switch delay are specified for FR2 HST scenarios.

The BS/UE performance requirements are discussed for FR2 HST scenarios, which will be further completed in the performance phase of this WI.

References

List of related CRs: select "TSG Status = Approved" in:

https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=890060,890160,890260

[1]	R4-2206594, TR 38.854 v0.4.0, NR support for high speed train scenario in frequency range 2 (FR2), Nokia, Nokia Shanghai Bell, and Samsung.

[2]	RP-220317, Status report for WI NR support for high speed train scenario in frequency range 2 (FR2), Samsung.

| 860043 | NR Dynamic spectrum sharing (DSS) | NR_DSS |  | RP-211345 | Ericsson |
| --- | --- | --- | --- | --- | --- |
| 860143 | Core part: NR DSS | NR_DSS-Core | R1 | RP-211345 | Ericsson |

Summary based on the input provided by Ericsson in RP-220464.

Dynamic spectrum sharing (DSS) provides a very useful migration path from LTE to NR by allowing LTE and NR to share the same carrier. DSS was included already in Rel-15 and further enhanced in Rel-16. As the number of NR devices in a network increases it is important that sufficient scheduling capacity for NR UEs on the shared carriers is ensured.

This is addressed by this WI, which introduces the support for cross-carrier scheduling from SCell to PCell/PSCell.

When cross-carrier scheduling from an SCell to sPCell is configured:

-	PDCCH on that SCell can schedule sPCell’s PDSCH and PUSCH,

-	PDCCH on the sPCell can schedule sPCell’s PDSCH and PUSCH but cannot schedule PDSCH and PUSCH on any other cell.

Only one SCell can be configured to be used for cross-carrier scheduling to sPCell.

The maximum number of monitoring candidates and non-overlapping CCEs for PDCCH monitoring (to schedule the sPCell) are split between the sPCell and the SCell used for scheduling the sPCell. The split is indicated via an RRC configured scaling factor.

Note:	sPCell refers to ‘Special Cell’. For Dual Connectivity operation the term Special Cell refers to the PCell of the MCG or the PSCell of the SCG, otherwise the term Special Cell refers to the PCell.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=860043,860143

[1]	RP-220463 – “Status report for WI: Core part: NR Dynamic spectrum sharing (DSS)”, Ericsson, RAN#95e.

| 860052 | Enhanced eNB(s) architecture evolution for E-UTRAN and NG-RAN | LTE_NR_arch_evo_enh |  | RP-211409 | China Unicom |
| --- | --- | --- | --- | --- | --- |
| 860152 | Core part: LTE_NR_arch_evo_enh | LTE_NR_arch_evo_enh-Core | R3 | RP-211409 | China Unicom |

Summary based on the input provided by China Unicom in RP-220560.

This WI specifies the interface, reusing E1 interface, interconnecting an eNB-CP (control plane and L2/L1 part of an eNB) and an eNB-UP (user plane part of an eNB) within E-UTRAN, or interconnecting an ng-eNB-CU-CP (control plane part of an ng-eNB central unit) and an ng-eNB-CU-UP (user plane part of an ng-eNB central unit) within NG-RAN[1]. In Release 16, E1 interface was limited to support interconnecting a gNB-CU-CP (control plane part of the gNB central unit) and a gNB-CU-UP (user plane part of the gNB central unit) in NG-RAN.

In the WI, a split of eNB into an eNB-CP and an eNB-UP is defined for E-UTRAN, and a split of ng-eNB-CU into an ng-eNB-CU-CP and an ng-eNB-CU-UP is defined for NG-RAN. The eNB-CP hosts the RRC/ RLC/MAC/PHY and the control plane part of the PDCP protocol, and the eNB-UP hosts the user plane part of the PDCP protocol [3].  The split of ng-eNB-CU into an ng-eNB-CU-CP and an ng-eNB-CU-UP are defined for NG-RAN [4]. The ng-eNB-CU-CP hosts the RRC and the control plane part of the PDCP protocol of the ng-eNB-CU, and the ng-eNB-CU-UP hosts hosting the user plane part of the PDCP protocol and the SDAP protocol of the ng-eNB-CU. The E1 interface is used between an eNB-CP and an eNB-UP as shown in Figure 1, or between an ng-eNB-CU-CP and an ng-eNB-CU-UP as shown in Figure 2. The ng-eNB-CU-CP is connected with the ng-eNB-DU via the W1-C interface, while the ng-eNB-CU-UP is connected with the ng-eNB-DU through the W1-U interface.

The architectures in Figure 1 and 2 enable the following deployment scenarios.

•	An eNB may consist of an eNB-CP and multiple eNB-UPs.

•	The eNB-UP is connected to the eNB-CP, while one eNB-UP is connected to only one eNB-CP.

•	The eNB-CP and the eNB-UP terminates the UP interface used to convey E-UTRA or NR PDCP PDUs. NR user plane protocol, as defined in TS 38.425 [5], is used for this interface.

•	An ng-eNB may consist of an ng-eNB-CU-CP, one or more multiple ng-eNB-CU-UP(s) and one or more multiple ng-eNB-DU(s).

•	One ng-eNB-DU is connected to only one ng-eNB-CU-CP, while one ng-eNB-CU-UP is connected to only one ng-eNB-CU-CP.

•	One ng-eNB-DU can be connected to multiple ng-eNB-CU-UPs under the control of the same ng-eNB-CU-CP;

•	One ng-eNB-CU-UP can be connected to multiple ng-eNB-DUs under the control of the same ng-eNB-CU-CP;

•	An ng-eNB-CU-CP and an ng-eNB-CU-UP is connected via the E1 interface.

•	An ng-eNB-DU is connected to an ng-eNB-CU-CP via the W1-C interface, and to an ng-eNB-CU-UP via the W1-U interface.

Figure 1: Overall architecture for separation of eNB-CP and eNB-UP

Figure 2: Overall architecture for separation of ng-eNB-CU-CP and ng-eNB-CU-UP

The general aspects and principles for E1 interface is specified in TS 37.480 [6], and layer 1 of E1 is specified in TS 37.481 [7].The E1 signalling transport supporting for ng-eNB-CU-CP/ng-eNB-CU-UP, eNB-CP/eNB-UP, which is based on the SCTP/IP protocol stack, are described in TS 37.482 [8]. The E1 application protocol (E1AP) supporting for ng-eNB-CU-CP/ng-eNB-CU-UP, eNB-CP/eNB-UP is specified in TS 37.483 [9], including the relevant descriptions of  E1 interface management procedures and E1AP elements, which allow to setup the E1 interface, exchange the relevant configuration data and date usage report in MR-DC between ng-eNB-CU-CP and ng-eNB-CU-UP, or between eNB-CP and eNB-UP.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=860052,860152

[1]	RP-22xxxx, “Status report of Enhanced eNB(s) architecture evolution for E-UTRAN and NG-RAN”, China Unicom

[2]	R3-214487	CR to 36.401: Baseline CR for introducing Rel-17 Enhanced eNB Architecture Evolution	Ericsson, Huawei, Nokia, Nokia Shanghai Bell

[3]	R3-214488	Further discussions on logical entities and corresponding definitions	Huawei, Ericsson

[4]	TS 38.425 v16.3.0 NG-RAN; NR user plane protocol

[5]	TS 37.480 v1.1.0 E1 general aspects and principles

[6]	TS 37.481 v1.0.0  E1 layer 1

| 890062 | RF requirements enhancement for NR frequency range 1 (FR1) | NR_RF_FR1_enh |  | RP-220681 | Huawei |
| --- | --- | --- | --- | --- | --- |
| 890162 | Core part: RF requirements enhancement for NR FR1 | NR_RF_FR1_enh-Core | R4 | RP-220681 | Huawei |
| 890262 | Perf. part: RF requirements enhancement for NR FR1 | NR_RF_FR1_enh-Perf | R4 | RP-220681 | Huawei |

Summary based on the input provided by Huawei, HiSilicon in RP-220682.

This work item develops several enhancement aspects for UE RF FR1. New RF requirements, corresponding new feature groups and functionalities are introduced for the WI as follows:

-	Specification restrictions for SUL bands supporting UL MIMO are removed in both RAN4 and RAN2 specs

-	In Rel-16, switching period between case 1 and case 2 was introduced to enable enhancement on UL performance with 2Tx transmission on one UL carrier for inter-band UL CA, SA SUL and inter-band EN-DC. UE capability on uplinkTxSwithingPeriod is introduced as 35μs, 140μs and 210μs (210μs only for inter-band UL CA and SA SUL). Meanwhile, UE DL interruption is allowed when configured with difficult band combinations. In Rel-17, the feature is extended to dynamic Tx switching between 2CC 2Tx-2Tx switching, 3CC 1Tx-2Tx switching and 3CC 2Tx-2Tx switching. In addtion, capability to indicate whether UL-MIMO coherence is supported when dynamic Tx switching between 3CC (within 2 bands) 1Tx-2Tx switching and 2CC or 3CC (within 2 bands) 2Tx-2Tx switching are introduced.

-	RF Requirements for PC3 and PC2 intra-band UL contiguous CA with UL MIMO are specified. During the study in Rel-17, it was agreed that no explicitly indication of specific UE architectures in the spec, i.e. 23+23, 23+26, 26+26. Also the conclusion is reached that MPR for 1T PC2 is applied for PC2 UL contiguous CA w/o UL MIMO or TxD indication while MPR for 2T 23+23 is applied for PC2 UL contiguous CA with UL MIMO and/or TxD indication.

-	RF Requirements for PC3 and PC2 intra-band non-contiguous UL CA are specified, especially the MPR values are defined based on different UE architectures (indicated via supporting dualPA-Architecture or not).

-	SCell dropping solution as a WI objective for preventing transmission power dropping on the cell with lower priority for both FR1 and FR2 CA is also discussed. As the solutions may have spec impact to other WGs, also there is no consensus whether it should be considered as a solution to address the potential in-field issue, the only possibility is to drop the discussion in Rel-17. Meanwhile, there was some discussion on PCMAX,CA and PHRCA reporting, similar to SCell dropping solution, no further discussion is considered in Rel-17.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=890062,890162,890262

[1]	RP-220680, SR on NR_RF_FR1_enh, RAN#95e

[2] 	R4-2107847, Reply LS on Rel-17 uplink Tx switching, China Telecom

[3] 	R2-2203986, RRC configuration for UL Tx switching enhancement, Huawei, HiSilicon, China Telecom, Apple, CATT

[4] 	R2-2203987, stage 2 CR for UL Tx switching enhancement, Huawei, HiSilicon, China Telecom

[5] 	R2-2203998, Introduction of Rel-17 Tx switching enhancements, China Telecom, Huawei, HiSilicon, Apple, CATT

| 890061 | NR measurement gap enhancements | NR_MG_enh |  | RP-213658 | MediaTek |
| --- | --- | --- | --- | --- | --- |
| 890161 | Core part: NR and MR-DC measurement gap enhancements | NR_MG_enh-Core | R4 | RP-213658 | MediaTek |
| 890261 | Perf. part: NR and MR-DC measurement gap enhancements | NR_MG_enh-Perf | R4 | RP-213658 | MediaTek |

Summary based on the input provided by MediaTek inc., Intel Corporation in RP-220752.

the 3 objectives of this WI are: 1) Pre-configured MG pattern(s), 2) Multiple concurrent and independent MG patterns and 3) Network controlled small gap. Corresponding network RRC signalling and measurement requirements are specified in TS38.331 and TS38.133, respectively.

1)	Pre-configured MG pattern(s)

- Introduced 2 activation/de-activation mechanisms and corresponding UE capabilities to support these two mechanism: Network controlled mechanism, in which UE follows the 1-bit per-BWP indications in active servings cell and 1-bit indication in deactivated serving cells to decide the ON/OFF status of the pre-configured MG; UE autonomous mechanism, in which UE follows the defined rules in TS38.133 to decide the ON/OFF status of the pre-configured MG. If MG is not needed for all measurements, the pre-configured gap should be deactivated (OFF) ; Otherwise, activated (ON). Events that may trigger UE to re-check the ON/OFF status includes: [DCI/Timer based BWP switching, activation/de-activation of SCell(s), addition/removal of any measurement object(s), addition/release/change of a Scell under CA , BWP switching by RRC, initiation of LocationMeasurementIndication];

- Introduced additional delay for pre-configured MG activation/deactivation which is 5ms on top on the legacy procedure delay that may trigger pre-configured MG status change;

- Updated the corresponding UE requirements regarding gap interruption, measurement delay and L1 measurement impact.

2)	Multiple concurrent and independent MG patterns

- Introduced multiple gap configurations and corresponding UE capability

- Introduced a mandatory association between gap and dedicated use cases (e,g., PRS, SSB, CSI-RS, EUTRA) by indicting a gap ID in the measurement objective or MG configuration (for PRS only). So that UE’s measurement behaviour is well-defined, because UE is only required to perform the measurement associated to the gap during that gap occasion.

- Introduced the maximum supported concurrent gap patterns for per-FR gap incapable/capable UEs. For per-FR gap incapable UE, up to 2 concurrent gap patterns can be configured. For per-FR gap capable UE, up to 3 concurrent gap patterns can be configured, which up to 2 gaps in one FR.

- Introduced a definition for the proximity condition of colliding gap occasions. Upon colliding, UE drops the gap with a lower priority level which is configured by network. Data scheduling is resumed on dropped gap occasion.

- Updated the corresponding UE requirements regarding gap interruption, measurement delay and L1 measurement impact.

3)	Network controlled small gap (NCSG)

- Introduced a UE capability reporting based on RRCReconfigurationComplete and RRCResumeComplete messages (similar to NeedforGap). So that UE can report whether to support ‘no-gap-no-interruption’, ‘ncsg’ or ‘gap’ for each target band to be measured based on UE’s current CA configuration.

- Introduced 24 NCSG patterns with visible interruption (VIL1 and VIL2, which are 1ms for FR1 and 0.75ms for FR2) before and after the measurement length (ML). UE is expected to continue DL reception or UL transmission with serving cells during ML. A new MG timing advance 0.75ms was introduced correspondingly.

Figure 1: 24 NCSG patterns with visible interruption

- Introduced the UE behaviours for the cases when UE reports different capabilities on ‘no-gap-no-interruption’, ‘ncsg’ or ‘gap’ but with a different a network configuration (NCSG or legacy MG) which may not perfectly match UE’s reported capability.

- Introduced a new synchronization indication between the target NR band to be measured and a reference serving cell of UE to reduce the OFDM symbols restricted from data scheduling, when UE is incapable for simultaneous Tx/Rx or independent beamforming (FR2-specific).

- Introduced the corresponding UE requirements regarding gap interruption, scheduling restriction and measurement delay. Update the impact to L1 measurements.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=890061,890161,890261

[1]	RP-213350, “Status report for WI: NR and MR-DC measurement gap enhancements”, Rapporteur (MediaTek Inc., Intel Corporation)

| 860040 | Further enhancements on MIMO for NR | NR_feMIMO |  | RP-212535 | Samsung |
| --- | --- | --- | --- | --- | --- |
| 860140 | Core part: Further enhancements on MIMO for NR | NR_feMIMO-Core | R1 | RP-212535 | Samsung |
| 860240 | Perf. part: Further enhancements on MIMO for NR | NR_feMIMO-Perf | R4 | RP-212535 | Samsung |

Summary based on the input provided by Samsung in RP-220802.

This WI introduces enhanced specification support for several key aspects on multi-input multi-output (MIMO) operation where Rel-15 and 16 NR were found deficient in terms of signalling latency, overhead, as well as spectral efficiency and coverage.

First, although Rel-15/16 NR supports FR2 deployments via multi-beam operation, signalling latency and overhead especially for beam indication are high for common beam operation. This not only hampers deployments in high-speed scenarios, but also scenarios where the number of configured TCI states is large. In addition, it lacks the support for inter-cell operation, multi-panel UEs, and maximum permissible exposure (MPE) mitigation.

Second, although Rel-16 NR introduced mTRP support for PDSCH, it still lacks ample support for PDCCH, PUSCH, and PUCCH. In addition, enhancements for inter-cell mTRP and beam management aspects specific to mTRP are introduced along with features to facilitate high-speed train deployment on single frequency network (HST-SFN).

Third, SRS has been a useful tool to acquire UL or DL CSI since Rel-15. However, the triggering mechanism of aperiodic SRS limits its flexibility. Further, antenna switching SRS only supports up to 4 antennas, which limits the performance for use cases requiring larger than 4 receive antennas. In addition, along with the expansion of 5G deployment, the demand to further improve SRS capacity and coverage appears.

Fourth, although Rel-16 has enhanced the Rel-15 Type-II to allow improved performance-overhead trade-off, CSI enhancements designed for mTRP non-coherent JT and Type-II assuming FDD (angle-delay) reciprocity can be beneficial to support additional deployment scenarios.

Enhancements for multi-beam operation

In Rel-15/16, UL beam indication utilizes a different framework from DL beam indication. As DL and UL beam indication share similar characteristics, a unified TCI-based framework for DL and UL beam indication is introduced in Rel-17 where UL TCI is used to represent UL spatial relation. Depending on whether DL and UL share the same TCI (e.g. in relation to the beam correspondence assumption supported by the UE), a UE can be configured with either joint DL/UL TCI (where DL and UL share the same TCI state hence a single TCI state update applies to both) or separate DL/UL TCI (which includes the signalling of DL-only, UL-only, and DL+UL TCI state update).

Central to the Rel-17 TCI framework is the use of common beam for UE-dedicated PDCCH and the associated PDSCH intended to streamline multi-beam operation. While Rel-15/16 supports the use of common beam for UE-dedicated PDCCH and the associated PDSCH, beam indication is performed via MAC-CE. The latency and overhead associated with MAC-CE potentially detract the application of NR for high-speed scenarios in the FR2 regime. In Rel-17, such latency and overhead are reduced with the introduction of DCI-based beam indication using the existing DCI formats 1_1/1_2 (with and without DL assignments) via the TCI field. To ensure that the NW and the UE are aligned in terms of TCI state, an ACK mechanism for the beam indication is supported either via the ACK associated with the PDSCH (when beam indication is accompanied with DL assignment) or an ACK mechanism analogous to that used for SPS PDSCH release (when beam indication is not accompanied with DL assignment). When only one TCI state is activated via MAC-CE, the DCI-based beam indication is not used. In this case, beam indication is performed via MAC-CE.

The Rel-17 TCI framework also supports auxiliary features such as a common TCI state ID update using a common TCI state pool and/or reference CC for Carrier Aggregation, UL power control setting (including PL-RS) association with TCI state, as well as inter-cell beam management. Inter-cell beam management includes measurement and reporting of L1-RSRP as well as beam indication for TCI states associated with PCIs different from that of the serving cell. In addition, common beam operation can be configured to other signals such as non-UE dedicated reception associated with serving cell PCI, aperiodic CSI-RS, and aperiodic SRS.

To better support UEs equipped with multiple panels, UE reporting of the capability associated with the maximum number of SRS ports and its correspondence with CSI-RS or SSB resource indicators (CRI/SSB-RI) is introduced. This reporting is performed in conjunction with the supported beam reporting.

Due to adherence to the MPE (maximum permissible exposure) regulation, some UL coverage penalty is incurred as the UE ends up using a sub-optimal UL transmit beam. To alleviate this issue, some enhancement in the existing PHR report is introduced where beam-specific P-MPR along with the associated CRI/SSBRI is added into the MAC-CE-based PHR report.

Enhancements for multi-TRP operation

In Rel-17, PDCCH repetition is defined by explicit linkage between two search space sets. The two linked search space sets can be associated with corresponding CORESETs with different TCI states, hence, achieving beam-diversity for PDCCH transmission. In Rel-17, only intra-slot PDCCH repetition is supported, and also, PDCCH repetition is only supported for USS or Type3 CSS. In addition, the linkage is specified at the PDCCH candidate level by restricting configurations of two linked search space sets resulting in one-to-one mapping between monitoring occasions and between PDCCH candidates of the two linked search space sets. Two linked PDCCH candidates have the same aggregation level, same coded bits, and the same DCI payload. To avoid ambiguity at the UE, a reference PDCCH candidate is defined for various procedures such as timelines, PUCCH resource determination, PDSCH reception with mapping Type B or mapping Type A, determination of QCL assumption for PDSCH when TCI field is not present in DCI, etc. UE can report whether two blind decodes or three blind decodes are needed for two linked PDCCH candidates. In the case of three blind decodes, overbooking for PDCCH is enhanced accordingly. Furthermore, determination of two QCL-TypeD is specified for FR2 to support time-overlapping PDCCH repetitions. PDCCH repetition is supported also for cross-carrier scheduling through linking two search space sets in both scheduling cell and scheduled cell.

In Rel-17, to support multi-TRP PUCCH repetition, up to two sets of power control parameters in FR1 or up to two PUCCH-SpatialRelationInfo in FR2 can be activated per PUCCH resource or per PUCCH resource group via MAC-CE. In addition, multi-TRP PUCCH repetition can be configured by intra-slot PUCCH repetition as well as inter-slot PUCCH repetition for all PUCCH formats. Based on the number of activated PUCCH-SpatialRelationInfo or set of power control parameters for the scheduled PUCCH resource, dynamic switching between single-TRP PUCCH repetition and multi-TRP PUCCH repetition can be supported. Separate power control for multi-TRP PUCCH repetition is supported by two activated PUCCH-SpatialRelationInfo or two activated sets of power control parameters. Furthermore, up to two TPC field for PUCCH can be supported and each TPC field is applied for each closed loop index.

In Rel-17, multi-TRP PUSCH repetition is supported with up to two SRS resource sets with usage set to ‘codebook’ or ‘nonCodebook’. If UE is provided by two SRS resource sets with usage set to ‘codebook’ or ‘nonCodebook’, the second SRI field, second TPMI field (if CB-based PUSCH is supported), and second PTRS-DMRS association field are indicated by DCI format 0_1 or 0_2 for PUSCH transmission occasion(s) toward the TRP which is related to the second SRS resource set with usage set to ‘codebook’ or ‘nonCodebook’ for dynamic grant based PUSCH scheduling. In addition, the new DCI field is defined as ‘SRS resource set indicator’ with 2 bits to support dynamic switching between single-TRP PUSCH repetition (corresponding to codepoint ‘00’ and ‘01’) and multi-TRP PUSCH repetition (corresponding to codepoint ‘10’ and ‘11’). Separate power control for multi-TRP PUSCH repetition is supported by linking between two SRI fields and two sets of power control parameters via higher layer parameter. And also up to two TPC field for PUSCH can be supported and each TPC field is applied for each closed loop index. Furthermore, the aforementioned multi-TRP PUSCH repetition is also supported by configured grant type 1 and 2.

In Rel-16, multi-TRP PDSCH transmission is supported with two different mechanisms: single-DCI and multi-DCI. In Rel-17, multi-TRP PDSCH transmission is further extended to inter-cell operation. A UE can be configured with SSB associated with PCI which are different from serving cell PCI, known as additional PCI. At most 7 different additional PCI can be configured to the UE, and only one is activated for inter-cell multi-TRP operation. The additional PCI can be associated with one or more TCI states, and gNB can schedule PDSCH dynamically from either TRP by indicating TCI in DCI.

In Rel-17, beam reporting and BFR are enhanced for mTRP scenario specifically. For beam reporting enhancement, framework of Rel-15 group based beam reporting is extended to facilitate simultaneous transmission of multiple TRPs. To achieve that, one CSI resource setting including two resource sets corresponding to two TRPs can be configured. In a CSI-report, UE can report N (Nmax = {1, 2, 3, 4}) groups of simultaneously received beams, wherein each reported beam in one group corresponds to one TRP. Differential L1-RSRP reporting across all beam groups is supported, and 1-bit is added to indicate CMR set associated with the largest RSRP value in the first group. For BFR enhancements, both single-DCI and multi-DCI framework are supported. For multi-DCI, two explicit or implicit BFD-RS sets are introduced for two TRPs in one CC. BFRQ can be transmitted if any one of BFD-RS sets fails. BFRQ framework is enhanced based on Rel.16 SCell BFR. The maximum number of PUCCH-SR resources is extended to 2. After UE receives gNB’s response, per-TRP beam reset can be performed. When both BFD-RS sets in SpCell fail, CBRA based RACH transmission can be triggered. For single-DCI, only explicit BFD-RS set configuration can be supported.

Rel-17 defines two key approaches for frequency offset compensation in HST-SFN scenario: UE-based and TRP-based compensation schemes. For UE-based compensation (scheme A), UE receives additional reference signals transmitted by the TRPs in a non-SFN manner to facilitate more accurate frequency offset compensation. The corresponding non-SFN TRS configurations are provided to the UE by using two TCI states (containing reference to the TRS of two TRPs) using DCI and MAC signaling. The TRP-based compensation (scheme B) relies on frequency offset pre-compensation at the network side, where each TRP estimates the downlink frequency by using uplink signal, e.g., SRS, and compensates the downlink frequency per TRP prior to transmission. For TRP based pre-compensation, UE also receives two TRS transmitted by the TRPs in a non-SFN manner using two TCI states. However, since network pre-compensates the PDCCH and PDSCH by the difference of the frequency offsets observed between two TRPs, frequency offset tracking at the UE is performed using only one TRS transmitted by the reference TRP.

SRS enhancements

In NR Rel-15/16, aperiodic SRS is triggered by DCI to provide dynamic indication of SRS transmission. However, only one triggering offset value can be configured in RRC per SRS resource set. This limits the valid combinations of the location to send the triggering DCI and the location to transmit SRS, esp. considering TDD slot format, where not all slots are available to transmit SRS. To address this issue, Rel-17 introduces SRS triggering offset enhancement based on available slot definition. Up to 4 offset values can be configured in RRC per SRS resource set, where each value is defined as the number of available slots counting from a reference slot. Reference slot is defined as the slot indicated by legacy triggering offset. In DCI format 0_1/0_2/1_1/1_2, one new field “SRS offset indicator” (SOI) is added to select one available slot offset value from the configured ones. In addition, Rel-15/16 DCI format 0_1/0_2 cannot be indicated to trigger SRS only, i.e., without CSI request and without data. This limitation is removed in Rel-17 by allowing gNB to trigger SRS based on DCI format 0_1/0_2 without CSI request and without data.

To ensure performance for uses case with larger than 4 receive antennas, Rel-17 introduces antenna switching SRS to support 1T6R, 1T8R, 2T6R, 2T8R and 4T8R. In addition, to improve flexibility of antenna switching SRS, Rel-17 extends the P/SP configurations by supporting maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for antenna switching, and the aperiodic configuration by supporting 1 or 4 aperiodic SRS resource sets for 1T4R and 2 aperiodic resource sets for 1T2R/2T4R.

In order to enhance the capacity and coverage for SRS, the following three schemes are specified in Rel-17.

- Increased repetition: Rel-17 supports 8, 10, 12 and 14 consecutive repetition symbols in one slot in one SRS resource.

- Partial frequency sounding: Rel-17 supports to transmit SRS only in 1PFmSRS,BSRS contiguous RBs in one OFDM symbol, where mSRS,BSRS  indicates the number of RBs configured by BSRS and CSRS. It can be applied on both frequency hopping case and non-frequency hopping case. The scaling factor PF is configured by RRC per SRS resource, which can be 2 or 4. The start RB index of the 1PFmSRS, BSRS RBs in the mSRS, BSRS RBs is Noffset=kF+khopping mod PFPFmSRS, BSRS , where kF is configured by RRC with a value chosen from kF = {0, 1, …, PF-1}. khopping is same for all SRS occasions within a legacy FH period but can hop across legacy FH periods. When the hopping of khopping is not enabled, it is fixed to be 0 for all SRS symbols. When the hopping of khopping is enabled, one hopping pattern is supported for each PF value.

- Comb 8: Rel-17 supports comb-8 for SRS other than for positioning, where the maximum number of cyclic shifts is 6.

CSI for mTRP and FDD reciprocity

The Rel-17 eType-II port selection (PS) codebook assumes W1W2WfHcodebook structure as Rell-16. The key enhancement comes from exploiting angle-delay reciprocity in DL and UL which is applicable for both TDD and FDD. By doing so, spatial domain and frequency domain compression operation inherent in the Rel-16 eType-II PS codebook can be shifted toward the gNB thereby reducing UE computational burden. While the amplitude and phase per channel/ propagation path are generally not DL and UL reciprocal, the gNB can employ angle-delay information obtained from UL measurements to precode UE-specific CSI-RS. Therefore from CSI measurement perspective at the UE, a subset of CSI-RS ports based on beamformed CSI-RS resource are first selected by the UE and represented by W_1, frequency domain compression is represented by Wf  giving rise to up to two selected DFT vectors, and lastly linear combination coefficients are quantized in amplitude and phase by W2 with configurable compression factors up to 1 by removing negligible coefficients.

Moreover, to improve CSI measurement accuracy over NCJT transmission in Rel-16, Rel-17 NCJT CSI enhancement supports a joint channel measurement at the UE by configuring a CMR pair within the same CSI-RS resource set corresponding to two TRPs respectively. Therefore a CSI-RS resource set is divided as two CMR groups so that each CMR pair consists of one CMR from Group 1 and one CMR from Group 2. From CSI reporting perspective, two modes are supported, whereas Mode 1 is to report X (X=0,1,2) single-TRP CSI and one NCJT CSI versus Mode 2 is to report one single-TRP or NCJT CSI. For the reporting quantity of NCJT CSI (up to rank 4), the main differentiation from legacy releases is that only single CQI/CRI is reported per CMR pair by jointly considering the best companion PMI/RI from two CMRs simultaneously to mitigate TRP interference. Also to use CSI-RS resources and configurations efficiently, sharing mechanism can be enabled by the gNB to reuse a CMR pair (configured for NCJT CSI measurement) for single-TRP CSI measurement if needed.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=860040,860140,860240

| 920070 | UE RF requirements for Transparent Tx Diversity (TxD) for NR | NR_RF_TxD | R4 | RP-211940 | Qualcomm |
| --- | --- | --- | --- | --- | --- |
| 920170 | Core part: UE RF requirements for Transparent Tx Diversity (TxD) for NR | NR_RF_TxD-Core | R4 | RP-211940 | Qualcomm |
| 920270 | Perf. part: UE RF requirements for Transparent Tx Diversity (TxD) for NR | NR_RF_TxD-Perf | R4 | RP-211940 | Qualcomm |

Summary based on the input provided by Qualcomm in RP-220923.

UE requirements for transmission diversity with 2 antenna connectors were defined. Up to Rel-16 specification did not recognise a UE that needed power measured from two connectors to fulfil the power class.

For SRS antenna switching and ULFPTx, the corresponding sections were updated.

A capability to indicate if UE support requirements under suffix G were added to the Rel-16.

During this WI, the following were added:

-	Requirements for UE implementation with two antenna connectors active when it is configured for one logical antenna port for PC2 and PC1.5.

-	Requirements for UE with tx diversity for SRS antenna switching were clarified

-	Fallback DCI requirements with one logical port when UE supports ULFPTx, part of Rel-16 eMIMO WI. A UE indicating the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for a band shall meet the 1Tx MOP requirement for at least one antenna connector

-	Capability for UE to indicate if it implements tx diversity

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=920070,920170,920270

[1]	RP-220467, “Status report for WI: UE RF requirements for Transparent Tx Diversity (TxD) for NR; rapporteur: Qualcomm”, RAN4, 3GPP TSG RAN Meeting #95-e, Electronic Meeting, March 17 - 23, 2022

[2]	RP-220608, “TR 38.837 v1.0.0 UE RF requirements for Transparent Tx Diversity (TxD) for NR”, vivo, RAN4, 3GPP TSG RAN Meeting #95-e, Electronic Meeting, March 17 - 23, 2022

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 860047 | UE power saving enhancements for NR | NR_UE_pow_sav_enh |  | RP-212630 | MediaTek |
| 860147 | Core part: UE power saving enhancements for NR | NR_UE_pow_sav_enh-Core | R2 | RP-212630 | MediaTek |
| 860247 | Perf. part: UE power saving enhancements for NR | NR_UE_pow_sav_enh-Perf | R4 | RP-212630 | MediaTek |
| 860049 | Further Multi-RAT Dual-Connectivity enhancements | LTE_NR_DC_enh2 |  | RP-201040 | Huawei |
| 860149 | Core part: Further Multi-RAT Dual-Connectivity enhancements | LTE_NR_DC_enh2-Core | R2 | RP-201040 | Huawei |
| 860249 | Perf. part: Further Multi-RAT Dual-Connectivity enhancements | LTE_NR_DC_enh2-Perf | R4 | RP-201040 | Huawei |
| 860050 | Enhancements to Integrated Access and Backhaul (IAB) for NR | NR_IAB_enh |  | RP-211548 | Qualcomm |
| 860150 | Core part: Enhancements to IAB for NR | NR_IAB_enh-Core | R2 | RP-211548 | Qualcomm |
| 860250 | Perf. part: Enhancements to IAB for NR | NR_IAB_enh-Perf | R4 | RP-211548 | Qualcomm |
| 830021 | Study on Security for NR_IAB | FS_NR_IAB_Sec | S3 | SP-201016 | Rajavelsamy Rajadurai, Samsung, |
| 890055 | Further enhancement on NR demodulation performance | NR_demod_enh2 |  | RP-212636 | China Telecom |
| 890255 | Perf. part: NR_demod_enh2 | NR_demod_enh2-Perf | R4 | RP-212636 | China Telecom |
| 890057 | NR RRM further enhancement | NR_RRM_enh2 |  | RP-202874 | Apple |
| 890157 | Core part: Further RRM enhancement for NR and MR-DC | NR_RRM_enh2-Core | R4 | RP-202874 | Apple |
| 890257 | Perf. part: Further RRM enhancement for NR and MR-DC | NR_RRM_enh2-Perf | R4 | RP-202874 | Apple |
| 890059 | Further enhancements of NR RF requirements for frequency range 2 (FR2) | NR_RF_FR2_req_enh2 |  | RP-212092 | Nokia |
| 890159 | Core part: NR_RF_FR2_req_enh2 | NR_RF_FR2_req_enh2-Core | R4 | RP-212092 | Nokia |
| 890259 | Perf. part: NR_RF_FR2_req_enh2 | NR_RF_FR2_req_enh2-Perf | R4 | RP-212092 | Nokia |

Summary based on the input provided by XXX in xP-22xxxx.

# 16	Other NR-only improvements

| 911006 | NR Uplink Data Compression (UDC) | NR_UDC |  | RP-220140 | CATT |
| --- | --- | --- | --- | --- | --- |
| 911106 | Core part: NR_UDC | NR_UDC-Core | R2 | RP-220140 | CATT |

Summary based on the input provided by CATT in RP-220142.

This work item specifies NR Uplink Data Compression (UDC), i.e. uplink data can be compressed at the UE and can be decompressed at the NG-RAN node.

In this WI, DEFLATE based UDC solution is introduced which uses LTE UDC as baseline.

DEFLATE based UDC solution could achieve higher compression efficiency which would save more uplink resources and reduce the transmission latency. The NG-RAN node can configure the UE to use UDC or not. If UDC is configured for a DRB, ROHC or EHC is not used for that DRB. One byte UDC header is introduced to indicate whether the PDCP SDU is compressed by UDC or not, whether the compression buffer is reset or not, and 4 validation bits of checksum are used to indicate whether the compression and decompression buffers are synchronous. For each DRB, the maximum compression buffer is 8192 bytes. If there are errors or failures due to buffer mismatching, the NG-RAN node can send a PDCP control PDU for error notification to the UE, the UE may reset the compression buffer when such notification has been received.

Similar as for LTE UDC, to improve compression efficiency of the first packets, two types of pre-defined dictionary can be used for UDC. One is standard dictionary for SIP and SDP signalling as defined in RFC 3485, and another is operator defined dictionary. The NG-RAN node configures whether or which dictionary is used for a UDC DRB.

NR UDC can be applied to NR-DC, NE-DC and NGEN-DC scenarios. Also, NR UDC can be applied to split bearer type. For NR-DC, NE-DC and NGEN-DC scenarios, MN can send an indication to SN regarding the maximum number of UDC DRBs allowed to SN terminated bearer. Furthermore, NR UDC can also be applied to CU CP and UP splitting scenario, where corresponding configuration parameters can be carried via E1 interface.

NR UDC continuity is also supported, which is similar as the ROHC continuity mechanism.

UDC related capabilities are also defined for UEs. There are four capabilities defined: supporting basic UDC function, supporting standard dictionary, supporting operator defined dictionary, and supporting UDC continuity. If the UE supports operator defined dictionary, it shall report the version of the dictionary and the associated PLMN ID to assist the NG-RAN node to identify the dictionary stored by the UE. The NG-RAN node configures UDC according to the signalled UE capabilities.

References

List of related CRs: select "TSG Status = Approved" in:

https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=911006,911106

[1]	RP-220141	Status Report for WI: NR Uplink Data Compression (UDC)	CATT

| 910096 | Study on band combination handling in RAN4 | FS_NR_ENDC_combo_rules | R4 | RP-211146 | ZTE |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by ZTE Corporation in RP-220165.

5G NR have much more complex band combination configurations than previous generations, due to the number of bands, multiple numerologies, larger channel bandwidth, size of channel bandwidth set, etc.

RAN4 specifications have been reorganized with the scope of this Work Item, as to provide a clearer view of all specified combinations. This CA/DC band combination related rule collections will help within and outside 3GPP.

This SI covers:

-   Capture of the workflow on introduction of band combinations for block approval and introduce new template of band combination request sheets for basket WIs as to reduce the workload of the basket WI rapporteur.

-   Indication of the rules for band combinations not valid or not for block approval.

-   Collect agreements on the rules of specifying band combinations, and facilitate people’s understanding of the complex notations of CA/DC combinations. This includes:

•   Rules of: CA/DC combination denotation; grouping EN-DC, NE-DC and NR-DC configurations.

•   Guidelines on: the band edge relaxation for MOP; introduction of PC2 combinations; introduction of band combinations with intra-band ULCA in UL configuration.

-   Study optimization rules for Rel-17 band combinations and provide further possible optimization in Rel-18, such as:

•   Removal of the redundant SCS information for inter-band CA configuration tables in Rel-17.

•   Optimization of FR2 intra-band no-contiguous CA configuration table in Rel-17 with no sub-block column explicitly shown.

•   Optimization proposal of ΔTIB,c and ΔRIB,c tables for band combinations. A mix of rule-based and table-based approach is proposed in Rel-18.

•   Further optimization on the new template for NR inter-band CA and SUL configuration tables in Rel-18.

References

[1]	R4-2203987, 3GPP TR 38.862 V0.6.0, Study on band combination handling in RAN4.

[2] 	RP-220164, Status report for SI Study on band combination handling in RAN4.

[3] 	R4-2206440, Email discussion summary for [102-e][140] FS_BC_handling.

(No related CRs.)

| 900070 | NR repeaters | NR_repeaters |  | RP-212129 | Qualcomm |
| --- | --- | --- | --- | --- | --- |
| 900170 | Core part: NR repeaters | NR_repeaters-Core | R4 | RP-212129 | Qualcomm |
| 930250 | Perf. part: NR repeaters | NR_repeaters-Perf | R4 | RP-212129 | Qualcomm |

Summary based on the input provided by Qualcomm in RP-220544.

This work item defines NR repeaters. These repeaters are network nodes designed to supplement/extend the coverage provided by base stations by simply amplifying and forwarding the signals from the input port without performing any other signal processing.

Repeater types: RF requirements for repeater types 1-C and 2-O are defined. Repeater type 1-C covers the conducted requirements for FR1 while type 2-O covers the radiated requirements for FR2. Other types of repeaters are not covered in this Release.

Repeater classes: Different repeater classes were introduced to cover different deployment scenarios and are differentiated for DL and UL as follows:

DL classes:

-	Wide Area repeaters are characterised by requirements derived from Macro Cell scenarios with a repeater to UE minimum distance along the ground equal to 35 m.

-	Medium Range repeaters are characterised by requirements derived from Micro Cell scenarios with a repeater to UE minimum distance along the ground equal to 5 m.

-	Local Area repeaters are characterised by requirements derived from Pico Cell scenarios with a repeater to UE minimum distance along the ground equal to 2 m.

UL classes:

-	Wide Area repeaters are characterised by requirements derived from Macro Cell and/or Micro Cell scenarios.

-	Local Area repeaters are characterised by requirements derived from Pico Cell and/or Micro Cell scenarios.

Repeater Operating Bands: NR repeater is designed to operate in the operating bands in FR1 and FR2-1 which are defined in TS 38.104. New bands added to 38.104 which are in these frequency ranges will automatically be applicable to the repeaters also.

TDD Operation: NR repeaters specifications also cover operation in the TDD bands. In these bands, the repeaters are assumed to be synchronized to the base station in whose coverage they are deployed follow the UL/DL frame configuration that the base station is using.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=900070,900170,930250

[1]	TS 38.106: NR repeater radio transmission and reception

[2]	TS 38.114: Repeaters ElectroMagnetic Compatibility (EMC)

[3]	TS 38.115-1: Repeater conformance testing - Part 1: Conducted conformance testing

[4]	TS 38.115-2: Repeater conformance testing - Part 2: Radiated conformance testing

[5]	TS 33.180: “Security of the Mission Critical (MC) service; (Release 17)”

| 911008 | NR QoE magement and optimizations for diverse services | NR_QoE |  | RP-211406 | China Unicom |
| --- | --- | --- | --- | --- | --- |
| 911108 | Core part: NR QoE management and optimizations for diverse services | NR_QoE-Core | R3 | RP-211406 | China Unicom |

Summary based on the input provided by China Unicom in RP-220556.

This WI specifies the NR QoE functionality.

The QoE Measurement Collection function enables the collection of application layer measurements from the UE. The measurements are supported for Streaming services, MTSI services, and VR services. Both signalling based and management based QoE measurement collection are supported. In this Release, basic application layer measurement collection mechanism is supported, and RAN visible QoE measurement mechanism and metrics are supported.

In the WI, the feature of QoE Measurement Collection function is activated in the NG-RAN either by direct configuration from the OAM system (management-based activation), or by signalling from the OAM via the Core Network (signalling-based activation), using UE-associated signalling. One or more QoE measurement collection jobs can be activated at a UE per service type. The application layer measurement configuration and measurement reporting is supported in RRC_CONNECTED state only. The QoE Measurement Collection deactivation permanently stops all or some of QoE measurement collection jobs towards a UE, resulting in the release of the corresponding QoE measurement configuration(s) in the UE. When a service is provided within a configured slice, the QoE Measurement Collection for this service type could also be configured together with the corresponding slice scope, so that the user experience of this service could also be evaluated on a per-slice basis.

The QoE Measurement Collection pause/resume procedure is used to pause/resume the reporting for all QoE reports or to pause/resume QoE reporting per QoE configuration in a UE for RAN overload situation. The gNB can send a downlink RRC message to temporarily stop the application layer measurement reports associated to one or multiple QoE configurations from UE to the network.

The QoE Measurement Collection continuity for intra-system intra-RAT mobility is supported in R17, with the area scope parameters configured by the OAM. The NG-RAN node is responsible for keeping track and identify whether the UE is inside or outside the area scope. A UE should continue an ongoing measurement even if it leaves the area scope, unless the NG-RAN node indicates to the UE to release the QoE configuration.

RAN visible QoE measurements are configured by the NG-RAN node, where a subset of QoE metrics is reported from the UE as an explicit IE to NG-RAN node. RAN visible QoE measurements (e.g., RAN visible QoE metrics, RAN visible QoE values) could be utilized by the NG-RAN node for network optimization. RAN visible QoE measurements are supported for the DASH streaming and VR service.

Radio-related measurements may be collected via immediate MDT for the supported services. The MCE/TCE performs the correlation of the immediate MDT results and the QoE measurement results collected at the same UE.

The following alignments are supported in this Release:

- Alignment between a signalling-based QoE measurement and a signalling-based MDT measurement.

- Alignment between a management-based QoE measurement and a management-based MDT measurement.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=911008,911108

[1]	RP-220554, Status report of SR for WI on NR QoE, China Unicom

| 900061 | NR coverage enhancements | NR_cov_enh |  | RP-211566 | China Telecom |
| --- | --- | --- | --- | --- | --- |
| 860036 | Study on on NR coverage enhancements | FS_NR_cov_enh | R1 | RP-200861 | China Telecom |
| 900161 | Core part: NR_cov_enh | NR_cov_enh-Core | R1 | RP-211566 | China Telecom |
| 900261 | Perf. part: NR_cov_enh | NR_cov_enh-Perf | R4 | RP-211566 | China Telecom |

Summary based on the input provided by China Telecom in RP-220564.

Coverage is one of the key factors that an operator considers when commercializing cellular communication networks due to its direct impact on service quality as well as CAPEX and OPEX. The Rel-17 study item 860036 “Study on NR coverage enhancements” evaluated the baseline performance and identified the potential bottleneck channels for both FR1 and FR2 [1]. This work item [2] specifies enhancements for PUSCH, PUCCH and Msg3 PUSCH, including enhancements on PUSCH repetition Type A, TB processing over multiple slots PUSCH, DMRS bundling for PUSCH/PUCCH, dynamic PUCCH repetition factor indication and Type A PUSCH repetitions for Msg3.

The following key functionalities are introduced as part of the Work Item:

Enhancements on PUSCH repetition Type A

For PUSCH repetition Type A, the maximum number of repetitions is increased up to 32. The increased maximum number of repetitions is applicable to PUSCH repetition Type A scheduled by DCI format 0_1 and DCI format 0_2 as well as PUSCH repetition Type A with Type 1 and Type 2 configured grant. In addition, PUSCH repetition Type A supports the repetitions counted based on available slots, and the maximum of repetitions for counting based on available slots is 32. When the counting based on available slots is enabled, a UE follows the 2-step procedure to perform PUSCH transmissions: in the first step, the UE determines available slots for K PUSCH repetitions; in the second step the UE determines whether to drop each of the K PUSCH repetition or not according to PUSCH dropping rules, where the PUSCH repetition is still counted in the K repetitions even if it is dropped.

TB processing over multiple slots PUSCH (TBoMS)

A single Transport Block (TB) using a resource allocated over multiple slots is introduced for PUSCH except Msg3. A UE scheduled to perform a TBoMS transmission, via either dynamic or configured grant (Type 2), uses the resource allocated across multiple slots to calculate the transport block size (TBS) for the transmission. Advantages of this approach as compared to single-slot PUSCH operations are in the form of lower effective coding rate and/or energy per resource element (EPRE) increase, both advantages leading to coverage enhancement. A TBoMS transmission can be performed w/ or w/o repetition, where a TBoMS transmission w/o repetition is referred to as single TBoMS transmission. The number of slots allocated for TBoMS is counted based on available slots. The UE transmits the TB across the slots determined as available for the TBoMS transmission, applying the same symbol and PRBs allocation in each slot, regardless of whether TBoMS is scheduled w/ or w/o repetition. Collision handling rules of Type A PUSCH repetition apply for TBoMS. TBS of a single TBoMS transmission is calculated using the resource in all the slots allocated for the single TBoMS transmission. The maximum supported TBS for TBoMS does not exceed legacy maximum supported TBS in Rel-15/16, for the same number of layers. TBoMS supports the transmission of only one layer and one code block. A single redundancy version (RV) is used for the transmission of a single TBoMS, irrespective of the number of slots allocated for a single TBoMS transmission, as per Figure 1. RV cycling is used across repetitions of a single TBoMS, as per Figure 2 . Rate Matching (RM) for TBoMS is performed per slot.

| 0 | 1 | 2 | 3 | Slot number |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | Used RV index per slot |

Figure 1. Example of single TBoMS transmission (w/o repetitions) where 4 slots are allocated for the single TBoMS.

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | Slot number |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 2 | 2 | 3 | 3 | 1 | 1 | Used RV index per slot |

Figure 2. Example of TBoMS transmission w/ repetitions, where 2 slots are allocated for the single TBoMS and 4 repetitions are configured.

DMRS bundling for PUSCH/PUCCH

DMRS bundling to enable improved channel estimation is introduced for PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, for PUSCH repetition Type A with configured grant, for PUSCH repetition Type B, for TB processing over multiple slots PUSCH and for PUCCH repetitions of PUCCH formats 1, 3, and 4. A UE can report the maximum duration, in number of consecutives slots, during which the UE is able to maintain power consistency and phase continuity under certain tolerance level. With the duration of the nominal TDWs (Time Domain Windows), not longer than the maximum duration, configured by gNB, one or multiple nominal TDWs can be determined for PUSCH transmissions or PUCCH repetitions. A nominal TDW consists of one or multiple actual TDWs. Events, which cause power consistency and phase continuity not to be maintained across PUSCH transmissions or PUCCH repetitions within the nominal TDW, are defined. An actual TDW is terminated in case an event occurs. A new actual TDW is created in response to semi-static events not triggered by DCI or MAC-CE. Whether a new actual TDW is created in response to dynamic events triggered by DCI is subject to UE capability. Frequency hopping and UL beam switching for multi-TRP operation are regarded as semi-static events. The UE shall maintain power consistency and phase continuity within an actual TDW across PUSCH transmissions or PUCCH repetitions.

Inter-slot frequency hopping with DMRS bundling to enable improved channel estimation within the same frequency hop is introduced for PUSCH repetition Type A, PUSCH repetition Type B, TBoMS and PUCCH repetitions. With the frequency hopping interval, N consecutive slots, configured by gNB, the UE performs inter-slot frequency hopping every N consecutive slots for PUSCH transmissions or PUCCH repetitions.

Dynamic PUCCH repetition factor indication

Dynamic repetition factor indication is introduced for PUCCH. This feature allows network to adjust PUCCH repetition factor in a flexible fashion based on channel conditions and traffic load. The mechanism to support this feature is an enhanced RRC configuration to configure different repetition factor per PUCCH resource. With this configuration, network can use existing PRI (PUCCH resource indicator) in DCI to point to a PUCCH resource where the associated repetition factor will be applied to the PUCCH transmission. By pointing to a different PUCCH resource via PRI, a different PUCCH repetition factor could apply, which implements the dynamic PUCCH repetition factor indication.

Type A PUSCH repetitions for Msg3

Up to 16 repetitions is introduced for a Msg3 PUSCH transmission scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI. This is beneficial to enhance the coverage on both NUL and SUL. After carrier selection and BWP selection during the RACH initialization procedure, a UE requests repetition of Msg3 PUSCH scheduled by RAR UL grant via separate PRACH resource when the RSRP of DL path-loss reference fulfils a configured threshold. If requested by the UE, gNB decides whether or not to schedule repetition of the Msg3 PUSCH transmission. If scheduled, the number of repetitions N is indicated by the 2 MSBs of the MCS field in the RAR UL grant or in the DCI format 0_0; and the MCS index used for the PUSCH transmission is indicated by the 2 LSBs of the MCS field in the RAR UL grant or by the 3 LSBs of the MCS field in the DCI format 0_0. The Msg3 PUSCH transmission is performed over N slots, which is counted based on available slots. Only inter-slot frequency hopping is supported if N>1 is indicated for the PUSCH transmission.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=900061,860036,900161,900261

[1]	TR 38.830, v17.0.0, “Study on NR coverage enhancements”, December, 2020.

[2]	RP-220563, “Status report for NR coverage enhancements”, China Telecom, RAN#95e, March 17th – 23rd, 2022.

| 911010 | Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC) | NR_FR1_TRP_TRS |  | RP-211158 | Vivo |
| --- | --- | --- | --- | --- | --- |
| 911110 | Core part: NR_FR1_TRP_TRS | NR_FR1_TRP_TRS-Core | R4 | RP-211158 | Vivo |
| 911210 | Perf. part: NR_FR1_TRP_TRS | NR_FR1_TRP_TRS-Perf | R4 | RP-211158 | Vivo |

Summary based on the input provided by vivo in RP-220606.

In Rel-15 and Rel-16, the UE FR1 transmit power and receiver sensitivity are tested by conducted methodology at the temporary antenna ports and it remains unknown what the actual performance of the UE would be in realistic network conditions with the UE antenna included. Radiated performance based on OTA testing is one of the most important characteristics to verify the entire UE performance under conditions more closely resembling the end user’s interaction with the device.

In order to ensure the good overall system performance, the requirements for NR UE TRP and TRS is important for consistent devices performance in the real NR networks which operate in the OTA manner. Unified requirements in 3GPP will provide authoritative guidance and will greatly promote the development of 5G industries.

This WI defines the test methodology to verify the NR UE TRP TRS performance for NR standalone (SA) and NR non-standalone (NSA) operation mode. The outcome is captured in a new technical report TR 38.834. Then 3GPP can specify the follow-up OTA requirements for FR1 UE based on the available test method.

A full package of test method under SA and NSA mode aiming to specify 3GPP TRP TRS OTA requirements is defined:

-	UE type: The test method covers device types including Smartphone (1st priority), Tablet, LEE and LME.

-	Usage scenarios: Talk mode using head & hand phantom for narrow devices between 56 mm and 72 mm and for wide devices with a width >72 mm and <92 mm; Browsing mode using hand phantom for narrow and wide phones; Free Space for devices not used in talk or browsing mode

-	Performance metrics: Definition of TRP and TRS for Anechoic-Chamber-based test methodology

-	UE positioning guidelines: UE positioning guidelines for Free space, Hand phantom only (Browsing mode), and Head and Hand phantom (Talk Mode); Both Wide Grip Hand and PDA Grip Hand positioning guidance (Wide Grip Hand for UE with Width >72mm and ≤92mm ; PDA Grip Hand for UE with Width ≥56mm and ≤72mm)

-	Test procedure for SA and EN-DC: Test setup for Single-antenna and multiple-antennas anechoic chambers; TRP TRS calibration procedure; Ripple test procedure for 30cm and 50cm, both theta-axis and phi-axis; SA TRP TRS performance measurement procedure; EN-DC TRP TRS performance measurement procedure (Only NR carrier measurement is needed); Minimum measurement distance of anechoic chambers

- UE configurations: TRP antenna configuration (TAS OFF with antenna locked at primary antenna); TRS antenna configuration (no specific setting); EN-DC configuration (For TRP UL configuration: the UL output power of LTE carrier should be set as a constant power of 10dBm, while measuring NR at maximum output power, i.e., with fixed p-MaxEUTRA-r15=10 dBm, and p-NR-FR1 not configured; For TRS UL configuration: The UL power configuration for LTE and NR is 50%-50% power splitting, i.e. For PC3, p-MaxEUTRA-r15=20 dBm, and p-NR-FR1= 20dBm; For PC2, p-MaxEUTRA-r15=23 dBm, and p-NR-FR1= 23dBm.)

- Measurement uncertainty assessment (RAN5): Measurement error uncertainty contribution descriptions; Preliminary example of uncertainty budget  (Expanded uncertainty for TRP hand only (browsing mode): 1.78 dB; Expanded uncertainty for TRP hand only (browsing mode): 2.20 dB)

- Test phantom definition: PDA Grip Hand; Wide Grip Hand; Head Phantom

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=

[1]	3GPP TR 38.834, Measurements of User Equipment (UE) Over-the-Air (OTA) performance for NR FR1; Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS) test methodology

[2]	Status Report: Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC), vivo

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 911009 | Rel-17 Power Class 2 UE for NR inter-band CA with or without SUL configurations with x (16>=x>2) bands DL and y (y=1, 2) bands UL | NR_UE_PC2_R17_CADC_SUL_xBDL_yBUL |  | RP-212182 | Huawei |
| 911109 | Core part: NR_UE_PC2_R17_CADC_SUL_xBDL_yBUL | NR_UE_PC2_R17_CADC_SUL_xBDL_yBUL-Core | R4 | RP-212182 | Huawei |
| 911209 | Perf. part: NR_UE_PC2_R17_CADC_SUL_xBDL_yBUL | NR_UE_PC2_R17_CADC_SUL_xBDL_yBUL-Perf | R4 | RP-212182 | Huawei |
| 911012 | High power UE (power class 1.5) for NR band n79 | NR_UE_PC1_5_n79 |  | RP-210843 | CMCC |
| 911112 | Core part: NR_UE_PC1_5_n79 | NR_UE_PC1_5_n79-Core | R4 | RP-210843 | CMCC |
| 911212 | Perf. part: NR_UE_PC1_5_n79 | NR_UE_PC1_5_n79-Perf | R4 | RP-210843 | CMCC |
| 930052 | UE Conformance - High power UE (power class 1.5) for NR band n79 | NR_UE_PC1_5_n79-UEConTest | R5 | RP-211979 | CMCC |
| 911013 | High power UE (power class 2) for NR band n34 | NR_UE_PC2_n34 |  | RP-210844 | CMCC |
| 911113 | Core part: NR_UE_PC2_n34 | NR_UE_PC2_n34-Core | R4 | RP-210844 | CMCC |
| 911213 | Perf. part: NR_UE_PC2_n34 | NR_UE_PC2_n34-Perf | R4 | RP-210844 | CMCC |
| 930053 | UE Conformance - High power UE (power class 2) for NR band n34 | NR_UE_PC2_n34-UEConTest | R5 | RP-211980 | CMCC |
| 911014 | High power UE (power class 2) for NR band n39 | NR_UE_PC2_n39 |  | RP-210845 | CMCC |
| 911114 | Core part: NR_UE_PC2_n39 | NR_UE_PC2_n39-Core | R4 | RP-210845 | CMCC |
| 911214 | Perf. part: NR_UE_PC2_n39 | NR_UE_PC2_n39-Perf | R4 | RP-210845 | CMCC |
| 930054 | UE Conformance - High power UE (power class 2) for NR band n39 | NR_UE_PC2_n39-UEConTest | R5 | RP-211981 | CMCC |
| 911015 | Introduction of FR2 FWA (Fixed Wireless Access) UE with maximum TRP of 23dBm for band n259 | NR_FR2_FWA_Bn259 |  | RP-210875 | SoftBank Corp., KDDI, NTT DOCOMO, Rakuten Mobile, Qualcomm |
| 911115 | Core part: NR_FR2_FWA_Bn259 | NR_FR2_FWA_Bn259-Core | R4 | RP-210875 | SoftBank Corp., KDDI, NTT DOCOMO, Rakuten Mobile, Qualcomm |
| 911215 | Perf. part: NR_FR2_FWA_Bn259 | NR_FR2_FWA_Bn259-Perf | R4 | RP-210875 | SoftBank Corp., KDDI, NTT DOCOMO, Rakuten Mobile, Qualcomm |
| 910097 | Study on optimizations of pi/2 BPSK uplink power in NR | FS_NR_Opt_pi2BPSK | R4 | RP-210910 | Huawei |
| 880078 | Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) requirements for NR UEs | NR_MIMO_OTA |  | RP-201800 | CAICT |
| 880178 | Core part: NR_MIMO_OTA | NR_MIMO_OTA-Core | R4 | RP-201800 | CAICT |
| 880278 | Perf. Part: NR_MIMO_OTA | NR_MIMO_OTA-Perf | R4 | RP-201800 | CAICT |
| 870064 | Introduction of FR2 FWA (Fixed Wireless Access) UE with maximum TRP (Total Radiated Power) of 23dBm for band n257 and n258 | NR_FR2_FWA_Bn257_Bn258 |  | RP-202565 | Softbank |
| 870164 | Core part: NR_FR2_FWA_Bn257_Bn258 | NR_FR2_FWA_Bn257_Bn258-Core | R4 | RP-202565 | Softbank |
| 870264 | Perf. Part: NR_FR2_FWA_Bn257_Bn258 | NR_FR2_FWA_Bn257_Bn258-Perf | R4 | RP-202565 | Softbank |
| 880092 | Adding channel bandwidth support to existing NR bands | NR_bands_R17_BWs |  | RP-212531 | Ericsson |
| 880192 | Core part: NR_bands_R17_BWs | NR_bands_R17_BWs-Core | R4 | RP-212531 | Ericsson |
| 880093 | Introduction of channel bandwidths 35MHz and 45MHz for NR FR1 | NR_FR1_35MHz_45MHz_BW |  | RP-211386 | Huawei |
| 880193 | Core part: NR_FR1_35MHz_45MHz_BW | NR_FR1_35MHz_45MHz_BW-Core | R4 | RP-211386 | Huawei |
| 880293 | Perf. Part: NR_FR1_35MHz_45MHz_BW | NR_FR1_35MHz_45MHz_BW-Perf | R4 | RP-211386 | Huawei |
| 880097 | SAR schemes for UE power class 2 (PC2) for NR inter-band Carrier Aggregation and supplemental uplink (SUL) configurations with 2 bands UL | NR_SAR_PC2_interB_SUL_2BUL |  | RP-212530 | China Telecom |
| 880197 | Core part: NR_SAR_PC2_interB_SUL_2BUL | NR_SAR_PC2_interB_SUL_2BUL-Core | R4 | RP-212530 | China Telecom |
| 880297 | Perf. Part: NR_SAR_PC2_interB_SUL_2BUL | NR_SAR_PC2_interB_SUL_2BUL-Perf | R4 | RP-212530 | China Telecom |
| 920065 | UE Conformance - SAR schemes for UE power class 2 (PC2) for NR inter-band Carrier Aggregation and supplemental uplink (SUL) configurations with 2 bands UL | NR_SAR_PC2_interB_SUL_2BUL-UEConTest | R5 | RP-211139 | China Telecom |
| 900062 | Support of reduced capability NR devices | NR_redcap |  | RP-211574 | Ericsson |
| 860035 | Study on support of reduced capability NR devices | FS_NR_redcap | R1 | RP-202704 | Ericsson |
| 900162 | Core part: NR_redcap | NR_redcap-Core | R1 | RP-211574 | Ericsson |
| 900262 | Perf. part: NR_redcap | NR_redcap-Perf | R4 | RP-211574 | Ericsson |
| 930018 | Architecture Enhancement for NR Reduced Capability Devices | ARCH_NR_REDCAP | S2 | SP-211100 | Aihua Li, China Mobile |
| 900064 | Additional NR bands for UL-MIMO power class 3 (PC3) in Rel-17 | NR_bands_UL_MIMO_PC3_R17 |  | RP-212184 | Huawei |
| 900164 | Core part: NR_bands_UL_MIMO_PC3_R17 | NR_bands_UL_MIMO_PC3_R17-Core | R4 | RP-212184 | Huawei |
| 900264 | Perf. part: NR_bands_UL_MIMO_PC3_R17 | NR_bands_UL_MIMO_PC3_R17-Perf | R4 | RP-212184 | Huawei |
| 900167 | Introduction of bandwidth combination set 4 (BCS4) for NR | NR_BCS4-Core | R4 | RP-202832 | Ericsson |
| 900068 | Downlink interruption for NR and EN-DC band combinations to conduct dynamic Tx Switching in Uplink | DL_intrpt_combos_TxSW_R17 |  | RP-210478 | China Telecom |
| 900168 | Core part: DL_intrpt_combos_TxSW_R17 | DL_intrpt_combos_TxSW_R17-Core | R4 | RP-202885RP-210478 | China Telecom |
| 900268 | Perf. part: DL_intrpt_combos_TxSW_R17 | DL_intrpt_combos_TxSW_R17-Perf | R4 | RP-202885RP-210478 | China Telecom |
| 900069 | High-power UE (power class 1.5) operation in NR bands n77 and n78 | HPUE_PC1_5_n77_n78 |  | RP-202912 | Qualcomm |
| 900169 | Core part: HPUE_PC1_5_n77_n78 | HPUE_PC1_5_n77_n78-Core | R4 | RP-202912 | Qualcomm |
| 930055 | UE Conformance - High-power UE (power class 1.5) operation in NR bands n77 and n78 | HPUE_PC1_5_n77_n78-UEConTest | R5 | RP-212581 | Verizon |
| 930056 | Increasing UE power high limit for CA and DC | Power_Limit_CA_DC | R4 | RP-212622 | Qualcomm |
| 930156 | Core part: Increasing UE power high limit for CA and DC | Power_Limit_CA_DC-Core | R4 | RP-212622 | Qualcomm |
| 930057 | Introduction of upper 700MHz A block E-UTRA band for the US | LTE_upper_700MHz_A | R4 | RP-212618 | Puloli |
| 930157 | Core part: Introduction of upper 700MHz A block E-UTRA band for the US | LTE_upper_700MHz_A-Core | R4 | RP-212618 | Puloli |
| 930257 | Perf. part: Introduction of upper 700MHz A block E-UTRA band for the US | LTE_upper_700MHz_A-Perf | R4 | RP-212618 | Puloli |
| 930058 | High power UE (power class 2) for one NR FDD band | NR_PC2_UE_FDD | R4 | RP-212633 | China Unicom |
| 930158 | Core part: High power UE (power class 2) for one NR FDD band | NR_PC2_UE_FDD-Core | R4 | RP-212633 | China Unicom |
| 930258 | Perf. part: High power UE (power class 2) for one NR FDD band | NR_PC2_UE_FDD-Perf | R4 | RP-212633 | China Unicom |

Summary based on the input provided by XXX in xP-22xxxx.

# 17	NR Carrier Aggregation

## 17.1	NR intra band Carrier Aggregation (itrabCA)

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 881005 | NR itrabCA for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y) | NR_CA_R17_Intra |  | RP-211757 | Ericsson |
| 881105 | Core part: NR_CA_R17_Intra | NR_CA_R17_Intra-Core | R4 | RP-211757 | Ericsson |
| 881205 | Perf. Part: NR_CA_R17_Intra | NR_CA_R17_Intra-Perf | R4 | RP-211757 | Ericsson |

Summary based on the input provided by XXX in xP-22xxxx.

## 17.2	NR inter band Carrier Aggregation (iterbCA)

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 900063 | High power UE for NR TDD intra-band Carrier Aggregation in frequency range FR1 | NR_intra_HPUE_R17 |  | RP-212180 | Huawei |
| 900163 | Core part: NR_intra_HPUE_R17 | NR_intra_HPUE_R17-Core | R4 | RP-212180 | Huawei |
| 900263 | Perf. Part: NR_intra_HPUE_R17 | NR_intra_HPUE_R17-Perf | R4 | RP-212180 | Huawei |
| 881006 | NR iterbCA/Dual Connectivity for 2 bands DL with x bands UL (x=1,2) | NR_CADC_R17_2BDL_xBUL |  | RP-212511 | ZTE |
| 881106 | Core part: NR_CADC_R17_2BDL_xBUL | NR_CADC_R17_2BDL_xBUL-Core | R4 | RP-212511 | ZTE |
| 881206 | Perf. part: NR_CADC_R17_2BDL_xBUL | NR_CADC_R17_2BDL_xBUL-Perf | R4 | RP-212511 | ZTE |
| 881007 | NR iterbCA for 3 bands DL with 1 band UL | NR_CA_R17_3BDL_1BUL |  | RP-212239 | CATT |
| 881107 | Core part: NR_CA_R17_3BDL_1BUL | NR_CA_R17_3BDL_1BUL-Core | R4 | RP-212239 | CATT |
| 881207 | Perf. part: NR_CA_R17_3BDL_1BUL | NR_CA_R17_3BDL_1BUL-Perf | R4 | RP-212239 | CATT |
| 881008 | NR iterbCA/Dual Connectivity for 3 bands DL with 2 bands UL | NR_CADC_R17_3BDL_2BUL |  | RP-212512 | ZTE |
| 881108 | Core part: NR_CADC_R17_3BDL_2BUL | NR_CADC_R17_3BDL_2BUL-Core | R4 | RP-212512 | ZTE |
| 881208 | Perf. part: NR_CADC_R17_3BDL_2BUL | NR_CADC_R17_3BDL_2BUL-Perf | R4 | RP-212512 | ZTE |
| 881009 | NR iterbCA for 4 bands DL with 1 band UL | NR_CA_R17_4BDL_1BUL |  | RP-211759 | Ericsson |
| 881109 | Core part: NR_CA_R17_4BDL_1BUL | NR_CA_R17_4BDL_1BUL-Core | R4 | RP-211759 | Ericsson |
| 881209 | Perf. part: NR_CA_R17_4BDL_1BUL | NR_CA_R17_4BDL_1BUL-Perf | R4 | RP-211759 | Ericsson |
| 881010 | NR iterbCA/Dual connectivity for DL 4 bands and 2UL bands | NR_CADC_R17_4BDL_2BUL |  | RP-212110 | Samsung |
| 881110 | Core part: NR_CADC_R17_4BDL_2BUL | NR_CADC_R17_4BDL_2BUL-Core | R4 | RP-212110 | Samsung |
| 881210 | Perf. part: NR_CADC_R17_4BDL_2BUL | NR_CADC_R17_4BDL_2BUL-Perf | R4 | RP-212110 | Samsung |
| 881011 | NR iterbCA for 5 bands DL with x bands UL (x=1, 2) | NR_CADC_R17_5BDL_xBUL |  | RP-212176 | Huawei |
| 881111 | Core part: NR_CADC_R17_5BDL_xBUL | NR_CADC_R17_5BDL_xBUL-Core | R4 | RP-212176 | Huawei |
| 881211 | Perf. part: NR_CADC_R17_5BDL_xBUL | NR_CADC_R17_5BDL_xBUL-Perf | R4 | RP-212176 | Huawei |
| 890054 | Rel-17 High power UE for NR inter-band Carrier Aggregation with 2 bands downlink and x bands uplink (x=1,2) | NR_PC2_CA_R17_2BDL_2BUL |  | RP-211831 | China Telecom |
| 890154 | Core part: NR_PC2_CA_R17_2BDL_2BUL | NR_PC2_CA_R17_2BDL_2BUL-Core | R4 | RP-211831 | China Telecom |
| 890254 | Perf. part: NR_PC2_CA_R17_2BDL_2BUL | NR_PC2_CA_R17_2BDL_2BUL-Perf | R4 | RP-211831 | China Telecom |
| 920066 | UE Conformance - Rel-17 High power UE for NR inter-band Carrier Aggregation with 2 bands downlink and x bands uplink (x=1,2) | NR_PC2_CA_R17_2BDL_2BUL-UEConTest | R5 | RP-211140 | China Telecom |

Summary based on the input provided by XXX in xP-22xxxx.

# 18	NR and LTE

| 860049 | Further Multi-RAT Dual-Connectivity enhancements | LTE_NR_DC_enh2 |  | RP-201040 |  |
| --- | --- | --- | --- | --- | --- |
| 860149 | Core part: Further Multi-RAT Dual-Connectivity enhancements | LTE_NR_DC_enh2-Core | R2 | RP-201040 |  |
| 860249 | Perf. part: Further Multi-RAT Dual-Connectivity enhancements | LTE_NR_DC_enh2-Perf | R4 | RP-201040 |  |

Summary based on the input provided by Huawei, HiSiliconin RP-220410.

This WI introduces the following enhancements:

-	for Carrier Aggregation, faster SCell activation

-	for EN-DC and for NR DC: a mechanism to deactivate and activate the NR Secondary Cell Group

-	support of inter-SN Conditional PSCell Change and of Conditional PSCell Addition

The enhancement to SCell activation allows faster usage of SCells. For traffic consisting primarily in short bursts, this allows deactivating SCells while there is not traffic, and reduce the UE power consumption, and having the additional carrier more quickly available when the next traffic burst appears.

The new SCG deactivation/activation mechanism allows keeping the NR SCG while saving UE power when the data traffic is lower and to activate the SCG again when needed due to increased traffic. This is also useful for bursty or variable data traffic.

Conditional PSCell Change was introduced in Rel-16 but limited to intra-SN PSCell change. This WI is adding the support inter-SN Conditional PSCell Change, thus extending the benefits to wider areas. In addition, Conditional PSCell Addition is supported, so that a PSCell can be added as soon as allowed by radio conditions.

Efficient SCell activation

To enable fast SCell activation when Carrier Aggregation is configured, for each configured SCell, the network can configure a number of aperiodic CSI-RS for tracking for fast SCell activation. A new MAC Control Element (CE) that indicates SCell activation can indicate the aperiodic CSI-RS for tracking that is activated by the network to assist the UE for activation of the SCell. This aperiodic CSI-RS can be used to assist Automatic Gain Control (AGC) and time/frequency synchronization.

SCG deactivation/activation

SCG deactivation/activation applies to an NR SCG, in EN-DC and in NR-DC.

UE behaviour while the SCG is deactivated

When the SCG is deactivated, UE activities on the SCG are reduced:

-	the UE does not transmit or receive any data via the SCG;

-	the UE does not perform/report any physical layer measurements;

-	all SCG SCells are deactivated.

However, other activities remain:

-	if configured to do so, the UE performs radio link monitoring and beam failure detection on the PSCell

-	the UE performs measurements for mobility, including measurements for SCG mobility configured by the SN;

-	the UE can exchange RRC signalling with the SN via the MCG as in Rel-15 (e.g. to report SCG radio link/beam failure, mobility measurement results);

-	in case of PSCell change, the UE does not perform random access towards the new PSCell if the SCG is to remain deactivated.

Data transmission while the SCG is deactivated

When the SCG is deactivated:

-	data can be transmitted or received via MCG bearers or via the MCG leg of split bearers;

-	SCG bearers can remain configured;

-	if there are uplink data to transmit on an SCG bearer, the UE indicates it to the network via RRC signalling.

SCG activation/deactivation

The network activates or deactivates the SCG by sending an RRC reconfiguration message to the UE.

For SCG activation, if the PSCell hasn't changed since the SCG deactivation, the timing advance is still valid and beam/radio link failure was not detected, it is possible to perform SCG activation without random access, i.e. the UE starts monitoring PDCCH and can receive downlink assignments and uplink grants. Otherwise, random access is necessary.

On the network side, SCG activation and SCG deactivation is coordinated between the MN and the SN using the existing procedure for SN addition and SN modification. Both the MN and the SN can request SCG activation or SCG deactivation and the other node can accept or reject the request.

Conditional PSCell Addition/Change

From the UE perspective, inter-SN conditional PSCell change is very similar to intra-SN conditional PSCell change, with a few main differences:

-	the conditional reconfiguration is received from the MCG;

-	the conditional reconfiguration can include a reconfiguration of both MCG and SCG;

-	the associated execution condition may refer to a conditional measurement configured by the MN (if initiated by the MN) or to a conditional measurement configured by the SN;

-	at CPC execution, the UE indicates to the MN which conditional reconfiguration was applied.

On the network side, there are a number of new procedures.

For instance, CPA and inter-SN CPC preparation requires exchanges between the MN, the SN and candidate target SN(s):

-	the initiating node (MN or SN) indicates a list of candidate target PSCells and associated execution conditions;

-	the MN provides this information to candidate target SN(s);

-	candidate target SN(s) provide conditional reconfigurations for all or a subset of the candidate target PSCells;

-	the MN transmits the conditional configurations to the UE, possibly with a MCG and SCG reconfiguration, as for any reconfiguration.

After CPA or CPC is prepared:

-	the initiating node can request candidate target SNs to update their conditional reconfigurations according to a new UE configuration, or to release them;

-	candidate target SN can initiate a modification or release already configured conditional reconfigurations.

References

List of related CRs: select "TSG Status = Approved" in:

https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=860049,860149,860249

[1]	RP-220409, Status report for WI: Multi-Radio Dual-Connectivity enhancements, Huawei, HiSilicon;

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 920072 | LTE/NR spectrum sharing in Band 34/n34 and Band 39/n39 | DSS_LTE_B34_NR_Bn34_LTE_B39_NR_Bn39 | R4 |  | CMCC |
| 920172 | Core part: LTE/NR spectrum sharing in Band 34/n34 and Band 39/n39 | DSS_LTE_B34_NR_Bn34_LTE_B39_NR_Bn39-Core | R4 | RP-211996 | CMCC |
| 920073 | High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands | LTE_NR_HPUE_FWVM | R4 | RP-212533 | Nokia |
| 920173 | Core part: High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands | LTE_NR_HPUE_FWVM-Core | R4 | RP-212533 | Nokia |
| 920273 | Perf. part: High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands | LTE_NR_HPUE_FWVM-Perf | R4 | RP-212533 | Nokia |
| 920074 | Rel-17 Dual Connectivity (DC) of x bands (x=1,2) LTE inter-band CA (xDL1UL) and 4 bands NR inter-band CA (4DL1UL) | DC_R17_xBLTE_4BNR_yDL2UL | R4 | RP-212606 | Huawei |
| 920174 | Core part: Rel-17 Dual Connectivity (DC) of x bands (x=1,2) LTE inter-band CA (xDL1UL) and 4 bands NR inter-band CA (4DL1UL) | DC_R17_xBLTE_4BNR_yDL2UL-Core | R4 | RP-212606 | Huawei |
| 920274 | Perf. part: Rel-17 Dual Connectivity (DC) of x bands (x=1,2) LTE inter-band CA (xDL1UL) and 4 bands NR inter-band CA (4DL1UL) | DC_R17_xBLTE_4BNR_yDL2UL-Perf | R4 | RP-212606 | Huawei |
| 911019 | Addition of MSD (Maximum Sensitivity Degradation) for inter-band EN-DC combinations (1 band LTE+1 band NR FR1) due to added channel bandwidths | ENDC_R17_1BLTE_1BNR_MS |  | RP-212187 | Huawei, HiSilicon |
| 911119 | Core part: ENDC_R17_1BLTE_1BNR_MS | ENDC_R17_1BLTE_1BNR_MSD-Core |  | RP-212187 | Huawei, HiSilicon |
| 911011 | Rel-17 Power Class 2 for EN-DC with x LTE bands + y NR band(s) in DL and with 1 LTE band +1 TDD NR band in UL (either x= 2, 3, y=1 or x=1, 2, y=2) | ENDC_PC2_R17_xLTE_yNR |  | RP-211760 | Ericsson |
| 911111 | Core part: ENDC_PC2_R17_xLTE_yNR | ENDC_PC2_R17_xLTE_yNR-Core | R4 | RP-211760 | Ericsson |
| 911211 | Perf. Part: ENDC_PC2_R17_xLTE_yNR | ENDC_PC2_R17_xLTE_yNR-Perf | R4 | RP-211760 | Ericsson |
| 930051 | UE Conformance - Power Class 2 for EN-DC with x LTE bands + y NR band(s) in DL and with 1 LTE band +1 TDD NR band in UL (either x= 2, 3, y=1 or x=1, 2, y=2) | ENDC_PC2_R17_xLTE_yNR-UEConTest | R5 | RP-211870 | Ericsson |
| 911018 | Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC | LTE_NR_Simult_RxTx |  | RP-211382 | Huawei |
| 911118 | Core part: LTE_NR_Simult_RxTx | LTE_NR_Simult_RxTx-Core | R4 | RP-211382 | Huawei |
| 911218 | Perf. part: LTE_NR_Simult_RxTx | LTE_NR_Simult_RxTx-Perf | R4 | RP-211382 | Huawei |
| 890052 | Rel-17 Dual Connectivity (DC) of 5 bands LTE inter-band CA (5DL/1UL) and 1 NR band (1DL/1UL) | DC_R17_5BLTE_1BNR_6DL2UL |  | RP-211195 | Samsung |
| 890152 | Core part: DC_R17_5BLTE_1BNR_6DL2UL | DC_R17_5BLTE_1BNR_6DL2UL-Core | R4 | RP-211195 | Samsung |
| 890252 | Perf. part: DC_R17_5BLTE_1BNR_6DL2UL | DC_R17_5BLTE_1BNR_6DL2UL-Perf | R4 | RP-211195 | Samsung |
| 890053 | Rel-17 Dual Connectivity (DC) of x bands (x=2, 3, 4) LTE inter-band CA (xDL/1UL) and 1 NR FR1 band (1DL/1UL) and 1 NR FR2 band (1DL/1UL) | DC_R17_xBLTE_2BNR_yDL3UL |  | RP-202542 | Samsung |
| 890153 | Core part: DC_R17_xBLTE_2BNR_yDL3UL | DC_R17_xBLTE_2BNR_yDL3UL-Core | R4 | RP-202542 | Samsung |
| 890253 | Perf. part: DC_R17_xBLTE_2BNR_yDL3UL | DC_R17_xBLTE_2BNR_yDL3UL-Perf | R4 | RP-202542 | Samsung |
| 880080 | Band combinations for concurrent operation of NR/LTE Uu bands/band combinations and one NR/LTE V2X PC5 band | NR_LTE_V2X_PC5_combos |  | RP-210588 | CATT |
| 880180 | Core part: NR_LTE_V2X_PC5_combos | NR_LTE_V2X_PC5_combos-Core | R4 | RP-210588 | CATT |
| 880280 | Perf. part: NR_LTE_V2X_PC5_combos | NR_LTE_V2X_PC5_combos-Perf | R4 | RP-210588 | CATT |
| 880094 | High power UE (power class 2) for EN-DC with 1 LTE band + 1 NR TDD band | ENDC_UE_PC2_R17_NR_TDD |  | RP-211853 | China Unicom |
| 880194 | Core part: ENDC_UE_PC2_R17_NR_TDD | ENDC_UE_PC2_R17_NR_TDD-Core | R4 | RP-211853 | China Unicom |
| 880294 | Perf. part: ENDC_UE_PC2_R17_NR_TDD | ENDC_UE_PC2_R17_NR_TDD-Perf | R4 | RP-211853 | China Unicom |
| 911000 | UE Conformance - ENDC_UE_PC2_R17_NR_TDD | ENDC_UE_PC2_R17_NR_TDD-UEConTest | R5 | RP-211420 | China Unicom |
| 880095 | LTE/NR spectrum sharing in Band 38/n38 | DSS_LTE_B38_NR_Bn38 |  | RP-201314 | Vodafone |
| 880195 | Core part: DSS_LTE_B38_NR_Bn38 | DSS_LTE_B38_NR_Bn38-Core | R4 | RP-201314 | Vodafone |
| 880096 | LTE/NR spectrum sharing in Band 40/n40 | DSS_LTE_B40_NR_Bn40 |  | RP-202084 | Reliance Jio |
| 880196 | Core part: DSS_LTE_B40_NR_Bn40 | DSS_LTE_B40_NR_Bn40-Core | R4 | RP-202084 | Reliance Jio |
| 880296 | Perf. part: DSS_LTE_B40_NR_Bn40 | DSS_LTE_B40_NR_Bn40-Perf | R4 | RP-202084 | Reliance Jio |
| 880098 | Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) | DC_R17_1BLTE_1BNR_2DL2UL |  | RP-212096 | CHTTL |
| 880198 | Core part: DC_R17_1BLTE_1BNR_2DL2UL | DC_R17_1BLTE_1BNR_2DL2UL-Core | R4 | RP-212096 | CHTTL |
| 880298 | Perf. part: DC_R17_1BLTE_1BNR_2DL2UL | DC_R17_1BLTE_1BNR_2DL2UL-Perf | R4 | RP-212096 | CHTTL |
| 880099 | Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL) | DC_R17_2BLTE_1BNR_3DL2UL |  | RP-212174 | Huawei |
| 880199 | Core part: DC_R17_2BLTE_1BNR_3DL2UL | DC_R17_2BLTE_1BNR_3DL2UL-Core | R4 | RP-212174 | Huawei |
| 890263 | Perf. part: DC_R17_2BLTE_1BNR_3DL2UL | DC_R17_2BLTE_1BNR_3DL2UL-Perf | R4 | RP-212174 | Huawei |
| 881000 | Dual Connectivity (DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL) | DC_R17_3BLTE_1BNR_4DL2UL |  | RP-211758 | Ericsson |
| 881100 | Core part: DC_R17_3BLTE_1BNR_4DL2UL | DC_R17_3BLTE_1BNR_4DL2UL-Core | R4 | RP-211758 | Ericsson |
| 881200 | Perf. part: DC_R17_3BLTE_1BNR_4DL2UL | DC_R17_3BLTE_1BNR_4DL2UL-Perf | R4 | RP-211758 | Ericsson |
| 881001 | Dual Connectivity (DC) of 4 bands LTE inter-band CA (4DL/1UL) and 1 NR band (1DL/1UL) | DC_R17_4BLTE_1BNR_5DL2UL |  | RP-212532 | Nokia |
| 881101 | Core part: DC_R17_4BLTE_1BNR_5DL2UL | DC_R17_4BLTE_1BNR_5DL2UL-Core | R4 | RP-212532 | Nokia |
| 881201 | Perf. part: DC_R17_4BLTE_1BNR_5DL2UL | DC_R17_4BLTE_1BNR_5DL2UL-Perf | R4 | RP-212532 | Nokia |
| 881002 | Dual Connectivity (DC) of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) | DC_R17_xBLTE_2BNR_yDL2UL |  | RP-211750 | LG Electronics |
| 881102 | Core part: DC_R17_xBLTE_2BNR_yDL2UL | DC_R17_xBLTE_2BNR_yDL2UL-Core | R4 | RP-211750 | LG Electronics |
| 881202 | Perf. part: DC_R17_xBLTE_2BNR_yDL2UL | DC_R17_xBLTE_2BNR_yDL2UL-Perf | R4 | RP-211750 | LG Electronics |
| 881003 | Dual Connectivity (DC) of x bands (x=1,2) LTE inter-band CA (xDL/xUL) and y bands (y=3-x) NR inter-band CA (yDL/yUL) | DC_R17_xBLTE_yBNR_3DL3UL |  | RP-211803 | ZTE |
| 881103 | Core part: DC_R17_xBLTE_yBNR_3DL3UL | DC_R17_xBLTE_yBNR_3DL3UL-Core | R4 | RP-211803 | ZTE |
| 881203 | Perf. part: DC_R17_xBLTE_yBNR_3DL3UL | DC_R17_xBLTE_yBNR_3DL3UL-Perf | R4 | RP-211803 | ZTE |
| 881004 | Dual Connectivity (DC) of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 3 bands NR inter-band CA (3DL/1UL) | DC_R17_xBLTE_3BNR_yDL2UL |  | RP-211805 | ZTE |
| 881104 | Core part: DC_R17_xBLTE_3BNR_yDL2UL | DC_R17_xBLTE_3BNR_yDL2UL-Core | R4 | RP-211805 | ZTE |
| 881204 | Perf. part: DC_R17_xBLTE_3BNR_yDL2UL | DC_R17_xBLTE_3BNR_yDL2UL-Perf | R4 | RP-211805 | ZTE |
| 881012 | band combinations for SA NR supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP) | NR_SUL_combos_R17 |  | RP-212178 | Huawei |
| 881112 | Core part: NR_SUL_combos_R17 | NR_SUL_combos_R17-Core | R4 | RP-212178 | Huawei |
| 881212 | Perf. part: NR_SUL_combos_R17 | NR_SUL_combos_R17-Perf | R4 | RP-212178 | Huawei |
| 900056 | UE Conformance - Rel-17 NR CA and DC; and NR and LTE DC Configurations | NR_CADC_NR_LTE_DC_R17-UEConTest | R5 | RP-211357 | Huawei |

Summary based on the input provided by XXX in xP-22xxxx.

# 19	NR new/modified bands

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 911016 | Introduction of 900MHz NR band for Europe for Rail Mobile Radio (RMR) | NR_RAIL_EU_900MHz |  | RP-211495 | UIC |
| 911116 | Core part: NR_RAIL_EU_900MHz | NR_RAIL_EU_900MHz-Core | R4 | RP-211495 | UIC |
| 911216 | Perf. part: NR_RAIL_EU_900MHz | NR_RAIL_EU_900MHz-Perf | R4 | RP-211495 | UIC |
| 911017 | Introduction of 1900MHz NR TDD band for Europe for Rail Mobile Radio (RMR) | NR_RAIL_EU_1900MHz_TDD |  | RP-211542 | UIC |
| 911117 | Core part: NR_RAIL_EU_1900MHz_TDD | NR_RAIL_EU_1900MHz_TDD-Core | R4 | RP-211542 | UIC |
| 911217 | Perf. part: NR_RAIL_EU_1900MHz_TDD | NR_RAIL_EU_1900MHz_TDD-Perf | R4 | RP-211542 | UIC |
| 860041 | Extending current NR operation to 71GHz | NR_ext_to_71GHz | R1 | RP-212637 | Qualcomm |
| 860141 | Core part: Extending current NR operation to 71GHz | NR_ext_to_71GHz-Core | R1 | RP-212637 | Qualcomm |
| 860241 | Perf. part: Extending current NR operation to 71GHz | NR_ext_to_71GHz-Perf | R4 | RP-212637 | Qualcomm |
| 860037 | Study on supporting NR from 52.6 GHz to 71 GHz | FS_NR_52_to_71GHz | R1 | RP-201838 | Intel |
| 870060 | Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz | FS_NR_IMT_param | R4 | RP-202276 | Ericsson |
| 870063 | Introduction of NR band n13 | NR_n13 | R4 | RP-200480 | Huawei |
| 870163 | Core part: Introduction of NR band n13 | NR_n13-Core | R4 | RP-200480 | Huawei |
| 870263 | Perf. part: Introduction of NR band n13 | NR_n13-Perf | R4 | RP-200480 | Huawei |
| 880082 | Introduction of NR 47GHz band | NR_47GHz_band | R4 | RP-212528 | Ericsson |
| 880182 | Core part: Introduction of NR 47GHz band | NR_47GHz_band-Core | R4 | RP-212528 | Ericsson |
| 880282 | Perf. part: Introduction of NR 47GHz band | NR_47GHz_band-Perf | R4 | RP-212528 | Ericsson |
| 880084 | Introduction of NR band 24 | NR_band_n24 | R4 | RP-211505 | Ligado Networks |
| 880184 | Core part: Introduction of NR band 24 | NR_band_n24-Core | R4 | RP-211505 | Ligado Networks |
| 880284 | Perf. part: Introduction of NR band 24 | NR_band_n24-Perf | R4 | RP-211505 | Ligado Networks |
| 890050 | Introduction of 6GHz NR licensed bands | NR_6GHz | R4 | RP-202844 | Huawei |
| 890150 | Core part: Introduction of 6GHz NR licensed bands | NR_6GHz-Core | R4 | RP-202844 | Huawei |
| 890250 | Perf. part: Introduction of 6GHz NR licensed bands | NR_6GHz-Perf | R4 | RP-202844 | Huawei |
| 920075 | Introduction of operation in full unlicensed band 5925-7125MHz for NR | NR_6GHz_unlic_full | R4 | RP-212302 | Apple |
| 920175 | Core part: Introduction of operation in full unlicensed band 5925-7125MHz for NR | NR_6GHz_unlic_full-Core | R4 | RP-212302 | Apple |
| 920275 | Perf. part: Introduction of operation in full unlicensed band 5925-7125MHz for NR | NR_6GHz_unlic_full-Perf | R4 | RP-212302 | Apple |
| 890051 | Introduction of lower 6GHz NR unlicensed operation for Europe | NR_6GHz_unlic_EU | R4 | RP-212625 | Nokia |
| 890151 | Core part: Introduction of lower 6GHz NR unlicensed operation for Europe | NR_6GHz_unlic_EU-Core | R4 | RP-212625 | Nokia |
| 890251 | Perf. part: Introduction of lower 6GHz NR unlicensed operation for Europe | NR_6GHz_unlic_EU-Perf | R4 | RP-212625 | Nokia |
| 900065 | Introduction of NR band n67 | NR_n67 | R4 | RP-202829 | Ericsson |
| 900165 | Core part: Introduction of NR band n67 | NR_n67-Core | R4 | RP-202829 | Ericsson |
| 900265 | Perf. Part: Introduction of NR band n67 | NR_n67-Perf | R4 | RP-202829 | Ericsson |
| 900066 | Introduction of NR band n85 | NR_n85 | R4 | RP-210707 | Ericsson |
| 900166 | Core part: Introduction of NR band n85 | NR_n85-Core | R4 | RP-210707 | Ericsson |
| 900266 | Perf. part: Introduction of NR band n85 | NR_n85-Perf | R4 | RP-210707 | Ericsson |
| 880083 | Introduction of 1.6 GHz NR supplemental uplink (SUL) band with same uplink frequency range of Band 24 | NR_SUL_UL_n24 |  | RP-210341 | Ligado Networks |
| 880183 | Core part: NR_SUL_UL_n24 | NR_SUL_UL_n24-Core | R4 | RP-210341 | Ligado Networks |
| 880283 | Perf. part: NR_SUL_UL_n24 | NR_SUL_UL_n24-Perf | R4 | RP-210341 | Ligado Networks |
| 880085 | Introduction of NR supplemental uplink (SUL) band 1880-1920MHz | NR_SUL_band_1880_1920MHz |  | RP-201363 | CMCC |
| 880185 | Core part: NR_SUL_band_1880_1920MHz | NR_SUL_band_1880_1920MHz-Core | R4 | RP-201363 | CMCC |
| 880285 | Perf. part: NR_SUL_band_1880_1920MHz | NR_SUL_band_1880_1920MHz-Perf | R4 | RP-201363 | CMCC |
| 880086 | Introduction of NR supplemental uplink (SUL) band 2300-2400MHz | NR_SUL_band_2300_2400MHz |  | RP-201364 | CMCC |
| 880186 | Core part: NR_SUL_band_2300_2400MHz | NR_SUL_band_2300_2400MHz-Core | R4 | RP-201364 | CMCC |
| 880286 | Perf. part: NR_SUL_band_2300_2400MHz | NR_SUL_band_2300_2400MHz-Perf | R4 | RP-201364 | CMCC |
| 900055 | UE Conformance - New Rel-17 NR licensed bands and extension of existing NR bands | NR_lic_bands_BW_R17-UEConTest | R5 | RP-202567 | Huawei |

Summary based on the input provided by XXX in xP-22xxxx.

# 20	LTE new/modified bands

| 911020 | New bands and bandwidth allocation for 5G terrestrial broadcast - part 1 | LTE_terr_bcast_bands_part1 | R1 | RP-211144 | EBU |
| --- | --- | --- | --- | --- | --- |
| 911120 | New bands and bandwidth allocation for 5G terrestrial broadcast - part 1 | LTE_terr_bcast_bands_part1 | R1 | RP-211144 | EBU |

Summary based on the input provided by EBU, Qualcomm Incorporated in RP-220445.

Before Rel-17, 5G terrestrial broadcast only supports the same system bandwidths as traditional LTE (1.4, 3, 5, 10, 15, 20MHz). Broadcast UHF spectrum, however, is channelized typically in channels of 6/7/8MHz (depending on the region). The objective of this work item [1] is to introduce support of PMCH bandwidths of 6/7/8MHz in MBMS-dedicated cells to enable deployment of 5G terrestrial broadcast in broadcast UHF spectrum.

The detailed operation to enable the above feature is as follows:

•	The eNB indicates in MIB a system bandwidth of 5MHz (25PRBs) or 3MHz (15PRBs).

•	In system information, the eNB indicates, per MBSFN area, whether the PMCH in that MBSFN area uses a bandwidth larger than the system bandwidth. The possible values for this bandwidth are 6MHz (30PRBs), 7MHz (35PRBs) and 8MHz (40PRBs).

•	The PMCH and MBSFN-RS are received with the indicated bandwidth.

A figure depicting the operation is shown below:

Figure 1: High level description of operation with 6/7/8MHz for PMCH

References

List of related CRs: select "TSG Status = Approved" in:

https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=911020,911120

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 880079 | Additional LTE bands for UE categories M1/M2/NB1/NB2 | LTE_bands_R17_M1_M2_NB1_NB2 |  | RP-211857 | Ericsson |
| 880179 | Core part: LTE_bands_R17_M1_M2_NB1_NB2 | LTE_bands_R17_M1_M2_NB1_NB2-Core | R4 | RP-211857 | Ericsson |
| 880279 | Perf. part: LTE_bands_R17_M1_M2_NB1_NB2 | LTE_bands_R17_M1_M2_NB1_NB2-Perf | R4 | RP-211857 | Ericsson |
| 880081 | Modification of LTE Band 24 Specifications to comply with updated regulatory emission limits | LTE_B24_mod |  | RP-211507 | Ligado Networks |
| 880181 | Core part: LTE_B24_mod | LTE_B24_mod-Core | R4 | RP-211507 | Ligado Networks |
| 880281 | Perf. part: LTE_B24_mod | LTE_B24_mod-Perf | R4 | RP-211507 | Ligado Networks |
| 920067 | UE Conformance - Modification of LTE Band 24 Specifications to comply with updated regulatory emission limits | LTE_B24_mod-UEConTest | R5 | RP-212497 | Ligado Networks |

Summary based on the input provided by XXX in xP-22xxxx.

# 21	LTE inter-band Carrier Aggregation (LTE iterbCA)

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 880087 | LTE iterbCA for 2 bands DL with 1 band UL | LTE_CA_R17_2BDL_1BUL |  | RP-211859 | Qualcomm |
| 880187 | Core part: LTE_CA_R17_2BDL_1BUL | LTE_CA_R17_2BDL_1BUL-Core | R4 | RP-211859 | Qualcomm |
| 880287 | Perf. part: LTE_CA_R17_2BDL_1BUL | LTE_CA_R17_2BDL_1BUL-Perf | R4 | RP-211859 | Qualcomm |
| 880088 | LTE iterbCA for 3 bands DL with 1 band UL | LTE_CA_R17_3BDL_1BUL |  | RP-212192 | Huawei |
| 880188 | Core part: LTE_CA_R17_3BDL_1BUL | LTE_CA_R17_3BDL_1BUL-Core | R4 | RP-212192 | Huawei |
| 880288 | Stopped - Perf. part: LTE iterbCA for 3 bands DL with 1 band UL | LTE_CA_R17_3BDL_1BUL-Perf | R4 | RP-202615 | Huawei |
| 880089 | LTE iterbCA Core for x bands DL (x=4, 5, 6) with 1 band UL | LTE_CA_R17_xBDL_1BUL |  | RP-211885 | Nokia |
| 880189 | Core part: LTE_CA_R17_xBDL_1BUL | LTE_CA_R17_xBDL_1BUL-Core | R4 | RP-211885 | Nokia |
| 880289 | Perf. part: LTE_CA_R17_xBDL_1BUL | LTE_CA_R17_xBDL_1BUL-Perf | R4 | RP-211885 | Nokia |
| 880090 | LTE iterbCA for 2 bands DL with 2 bands UL | LTE_CA_R17_2BDL_2BUL |  | RP-212194 | Huawei |
| 880190 | Core part: LTE_CA_R17_2BDL_2BUL | LTE_CA_R17_2BDL_2BUL-Core | R4 | RP-212194 | Huawei |
| 880290 | Perf. part: LTE_CA_R17_2BDL_2BUL | LTE_CA_R17_2BDL_2BUL-Perf | R4 | RP-212194 | Huawei |
| 880091 | LTE iterbCA for x bands DL (x= 3, 4, 5) with 2 bands UL | LTE_CA_R17_xBDL_2BUL |  | RP-211748 | LG |
| 880191 | Core part: LTE_CA_R17_xBDL_2BUL | LTE_CA_R17_xBDL_2BUL-Core | R4 | RP-211748 | LG |
| 880291 | Perf. part: LTE_CA_R17_xBDL_2BUL | LTE_CA_R17_xBDL_2BUL-Perf | R4 | RP-211748 | LG |

Summary based on the input provided by XXX in xP-22xxxx.

# 22	User plane improvements

| 820003 | Support of Immersive Teleconferencing and Telepresence for Remote Terminals | ITT4RT | S4 | SP-180985 | Ozgur Oyman, Intel |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Nokia Corporation in SP-220275.

This Work Item extends the functionality of Multimedia Telephony Service for IMS (MTSI) in TS 26.114 by adding the Virtual Reality (VR) unidirectional video transmission capability.

Earlier TS 26.114 was capable of handling real-time multimedia communications of traditional media (e.g., audio and video). The completed WI on ITT4RT enables, on top of the mentioned specification, new VR use cases and allows transmitting and receiving, in addition to traditional media, also uni-directional 360-degree video that can be viewed using Head Mounted Displays and 5G devices. This makes the end-user experience more compelling and immersive.

In addition, two more documents have been produced as part of this WI: TR 26.962 (Immersive Teleconferencing and Telepresence for Remote Terminals (ITT4RT) Operation and Usage Guidelines) and TR 26.862 (Immersive Teleconferencing and Telepresence for Remote Terminals (ITT4RT) Use Cases, Requirements and Potential Solutions).

The ITT4RT WI adds to TS 26.114 the following:

-	Support of still images, image sequences and still 360-degree background

-	Support of 360-degree video for H.265

-	Support of overlays on top of 360-degree video

-	Support of multiple video projection formats

-	Support for fisheye video

-	Support of camera calibration for Network-based Stitching

-	Support of picture packing for 360-degree video

-	Support of viewport dependent processing

-	Support of improved feedback for 360-degree video

-	Support of captured content replacement for screen sharing

-	Recommended audio mixing gains

-	Examples SDP offers and answers for 360-degree video.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=820003

[1]	TR 26.962. S4-220269

[2]	TR 26.862. S4-220323

| 900002 | Restoration of PDN Connections in PGW-C/SMF Set | RPCPSET | C4 | CP-203020 | LANDAIS, Bruno, Nokia |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Nokia in CP-220149.

This Work Item extends the functionality of Multimedia Telephony Service for IMS (MTSI) in TS 26.114 by adding the Virtual Reality (VR) unidirectional video transmission capability.

The work item defines PDN connection restoration procedures that enable to restore PDN connections in EPC after a PGW-C/SMF failure, restart or scale-in operation, by allowing to move PDN connections to a different PGW-C/SMF of the PGW-C/SMF set.

SMF scalability and resiliency can be supported for PDU sessions in 5GS by deploying an SMF set, i.e. a set of SMF instances that are functionally equivalent and inter-changeable and that share the same contexts (as defined in Release 16). This enables e.g. an AMF, PCF or UPF to reselect a different SMF in the same SMF set when the SMF serving a PDU session fails, restarts or is removed from the SMF set (scale-in operation), without interrupting the services and the PDN connectivity of the PDU session.

Inter-system mobility between 5GS and EPS relies on combo PGW-C/SMF. Combo PGW-C/SMF can be deployed in a PGW-C/SMF set. The work item defines PDN connection restoration procedures to restore the PDN connections served by a PGW-C/SMF, when a PGW-C/SMF in a PGW-C/SMF set fails, restarts or is removed from the PGW-C/SMF set. This allows to fully leverage the benefits of deploying PGW-C/SMF set by enabling:

-	to scale-in a PGW-C/SMF set without tearing down and re-establishing all the PDN connections of the PGW-C/SMF that is removed from the set;

-	an MME or ePDG to reselect a different PGW-C/SMF in the PGW-C/SMF set for an on-going PDN connection or a group of PDN connections, when the PGW-C/SMF that was serving the PDN connection or the group of PDN connections fails or restarts, and to maintain the services and PDN connectivity of the PDN connection or group of PDN connections; and

-	a PGW-C/SMF to instruct an MME, ePDG and/or PGW-U/UPF to use a new PGW-C/SMF for an on-going PDN connection or group of PDN connections, when e.g. a PCF or PGW-U/UPF reselects a different PGW-C/SMF from the PGW-C/SMF set.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=900002

| 830005 | Terminal Audio quality performance and Test methods for Immersive Audio Services | ATIAS | S4 | SP-190040 | Stéphane Ragot, Orange |
| --- | --- | --- | --- | --- | --- |
| 860012 | Handsets Featuring Non-Traditional Earpieces | HaNTE | S4 | SP-190989 | Schevciw, Andre, Qualcomm |
| 920008 | 5GMS AF Event Exposure | EVEX | S4 | SP-210374 | Lo, Charles, Qualcomm Inc., |
| 920009 | Edge Extensions to the 5G Media Streaming Architecture | 5GMS_EDGE | S4 | SP-210375 | Bouazizi, Imed, Qualcomm Inc. |
| 920010 | 5G Multicast-Broadcast User Service Architecture and related 5GMS Extensions | 5MBUSA | S4 | SP-210376 | TAN, PENG, TELUS |
| 920011 | 8K Television over 5G | 8K_TV_5G | S4 | SP-210381 | Thomas Stockhammer |

# 23	Signalling optimisations

| 880049 | Enhancement for the 5G Control Plane Steering of Roaming for UE in Connected mode | eCPSOR_CON |  | CP-210148 | Minokuchi, Atsushi, NTT DOCOMO |
| --- | --- | --- | --- | --- | --- |
| 850039 | Stage 1 of eCPSOR_CON | eCPSOR_CON | S1 | SP-190941 | Minokuchi, Atsushi, NTT DOCOMO |
| 880022 | Stage 3 (CT1) of eCPSOR_CON | eCPSOR_CON | C1 | CP-210148 | Al-Bakri Ban, NTT DOCOMO |
| 900036 | Stage 3 (CT4) of eCPSOR_CON | eCPSOR_CON | C4 | CP-210148 | Al-Bakri Ban, NTT DOCOMO |
| 920061 | Stage 3 (CT6) of eCPSOR_CON | eCPSOR_CON | C6 | CP-210148 | Al-Bakri Ban, NTT DOCOMO |

Summary based on the input provided by NTT DOCOMO in SP-220260.

Steering of roaming (SOR) allows the Home PLMN (HPLMN) operator to steer a UE to a visited PLMN (VPLMN) on which the HPLMN wants the UE to register, when the UE registers on another VPLMN. This capability may be needed for reasons such as reselection to a higher priority PLMN based on business arrangements (TS22.261 – clause 6.30.1).

Steering of Roaming (SOR) is used since GSM, and the requirements are provided in TS22.011.

In 5G, the Control Plane SOR solution (CP-SOR) was introduced using the same requirements in TS22.011.

In Rel-17, additional requirements specific for CP-SOR were provided in TS22.261 that allows the HPLMN to control the timing when a UE registered on a VPLMN, in automatic mode (see clause 3.1 of TS 23.122 [25]) and currently in CONNECTED mode, enters IDLE mode and initiates higher priority PLMN selection based on the type of ongoing communication.

Upon receiving CP-SOR information defined in Rel-15 and Rel-16, while the UE is in CONNECTED mode, the UE waits until it moves to idle mode before performing SOR and attempting to obtain service on a higher priority PLMN.

The UEs in 5G may stay in connected mode for a rather long time, whole day or longer, without going to idle mode. The HPLMN operator may have means to evaluate what is more convenient to provide the service for the user (e.g., based on their subscription profile, more efficient economically from wholesale perspective, allow users to use dedicated "economical" retail plans on specific VPLMN, etc.) and decide which VPLMN is more appropriate for the user to register on. Therefore, new requirements are introduced to allow the HPLMN to enforce the interruption of the ongoing sessions for the sake of performing SOR and moving the UE to another VPLMN to obtain service on a higher priority PLMN.

This feature introduces means to send additional steering of roaming information, called steering of roaming connected mode control information (SOR-CMCI) that enables the HPLMN to control the timing of a UE in connected mode to move to idle mode to perform the steering of roaming. The UE shall support the SOR-CMCI. The support and use of SOR-CMCI by the HPLMN is based on the HPLMN's operator policy.

The SOR-CMCI is provided by the HPLMN operator using different means, a specific application function (SOR-AF) or the information can be stored in the repository system (UDR). This information can be provided to the UE during registration procedure or after the registration (triggered by some parameters in the SOR-AF or the UDR).

The UE's type of ongoing communication / session(s) should be taken into consideration. The high priority services and emergency services are exempted from being interrupted (i.e. this feature is not applicable).

The new SOR related information is to be exchanged between the HPLMN UDM and the UE in a secured manner therefore the HPLMN data management application function ensures the security of the information using the Authentication Server Function (AUSF). The SOR information is transparent for the VPLMN, and if the VPLMN manipulates the information then the security of the information will break. To ensure the security of the information, the HPLMN may request the UE to provide an acknowledgement that the information is received securely.

This feature is optional for the HPLMN. This means that if this optional feature is not selected by the HPLMN operator, then there shall be no impact on the existing SOR functionality.

This feature is applicable only when the UE is in automatic network selection mode (see TS 23.122).

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=880049,850039,880022,900036,920061

[1]	TS 22.011: "Service accessibility".

[2]	TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[4]	TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

| 910044 | Same PCF Selection For AMF and SMF | TEI17_SPSFAS |  | SP-200447 | Heng, Nie, China Telecom |
| --- | --- | --- | --- | --- | --- |
| 880016 | Stage 2 for TEI17_SPSFAS | TEI17_SPSFAS | S2 | SP-200447 | Heng, Nie, China Telecom |
| 910003 | CT aspects on TEI17_SPSFAS | TEI17_SPSFAS | ct | CP-211184 | Yue Sun, China Telecom |
| 910045 | CT3 aspects on TEI17_SPSFAS | TEI17_SPSFAS | C3 | CP-211184 | Yue Sun, China Telecom |
| 910046 | CT4 aspects on TEI17_SPSFAS | TEI17_SPSFAS | C4 | CP-211184 | Yue Sun, China Telecom |

Summary based on the input provided by China Telecom in SP-220261.

Binding the Access and Mobility Management (AM) and the Session Management (SM) policies provides several benefits, such as the change of  RAT/Frequency Selection Priority (RFSP) values for a UE when the quota for the Data Network Name (DNN) for enhanced Mobile Broadband (eMBB) reaches the limit, or simplify the network deployment. A simple way to achieve such binding is to select the same Policy Control Function (PCF) for the AMF and the SMF.

In Rel 15 and 16, the same PCF selection for the AMF and the SMF is achieved by the mechanism of AMF optionally forwarding the selected PCF to SMF instance(s) during the PDU Session Establishment procedure(s), and the SMF may select the received PCF for SM Policy Control based on local policies. But it is applicable only in the UE initially accessing via 5GS scenario.

This Rel 17 work item specifies a mechanism to achieve the same PCF selection for AMF and SMF in the EPS to 5GS mobility scenario, where the PCF selection for AMF may be performed after the PCF selection for SMF/PGW-C. In this WI, subscription data is expanded to assist the same PCF selection, to offer more flexibility for operation of corresponding service.

The mechanism in this WI is based on the PGW-C+SMF registration of the selected PCF for SM Policy Control into the UDM+HSS. When performing PCF selection, based on subscription data for the PCF selection received from the UDM, the AMF determines that the same PCF for AMF and the SMF serving a specific DNN and/or S-NSSAI needs to be selected, then it selects the PCF ID serving the specific DNN and/or S-NSSAI received from UDM before initiating the Establishment of AM Policy Association. It is applicable for EPS to 5GS Mobility.

For the scenario of pure 5GS, based on subscription data for the PCF selection received from the UDM, the AMF determines that the same PCF for AMF and SMF serving a specific DNN and/or S-NSSAI needs to be selected, then it forwards the PCF ID for AM policies to the SMF and may additionally indicate to the SMF that the same PCF should be selected for SM Policy Control.

References

List of related CRs: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=910044,880016,910003,910045,910046

[1]	TS 23.501: “System architecture for the 5G System (5GS)”

[2]	TS 23.502: “Procedures for the 5G System; Stage 2”

[3]	TS 29.502: “5G System; Session Management Services; Stage 3”

[4]	TS 29.503: “5G System; Unified Data Management Services; Stage 3”

[5]	TS 29.513: “5G System; Policy and Charging Control signalling flows and QoS parameter mapping; Stage 3”

| 900037 | Enhancement of Inter-PLMN Roaming | EoIPR | C4 | CP-203021 | Caixia Qi, Huawei |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Huawei in CP-220152.

The work item defines some enhancements on the interfaces between vNSSF and hNSSF, vNRF and hNRF, vSEPP and hSEPP in stage3 for inter PLMN roaming in 5GC scenario.

Inter PLMN roaming is deployed for a user roaming into a vPLMN which has a roaming agreement with the UE's hPLMN. Some enhancements in stage3 are specifies with regards to Inter PLMN roaming scenarios:

-	The NSSF in the vPLMN can invoke the Nnssf_NSSelection service provided by the NSSF in the hPLMN, TS 29.531 specification for the interface between vNSSF and hNSSF is updated to specify the condition/procedure to support the communication.

-	Multiple NRFs can be deployed in a vPLMN and/or an hPLMN. TS 29.510 specification for the interface between vNRF and hNRF is updated to support the modification or removal of the subscription in the hNRF if multiple NRFs are deployed in hPLMN.

-	TS 29.573 specification for the interface between vSEPP and hSEPP is updated based on the requirements from GSMA and SA3 on the enhancements to the roaming.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=900037

| 940003 | Enhancement on the GTP-U entity restart | EGTPUR | C4 | CP-213097 | YONG YANG, Ericsson |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Ericsson in CP-220212.

A remote GTP-U restart will lead massive amount of signalling in the network (for reporting GTP-U Error Indication and PFCP Session Modification Signalling messages) over Sx/N4 interface.

To avoid that, this work item specifies following enhancements:

-		procedures to detect and report the peer GTP-U entity restart;

-		protocol support in GTP-U to enable to detect peer GTP-U entity restart;

-		protocol support in PFCP to enable User Plane function to report the peer GTP-U entity restart to the Control Plane function.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=940003

| 880032 | Packet Flow Description (PFD) management enhancement | pfdManEnh | C3 | CP-210183 | Xiaoyun Zhou, Huawei |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Huawei in CP-210183.

An optimize solution to reduce the load of signalling for pull mode in EPS and 5GS is defined. A notification push solution for push mode in 5GS is defined.

In stage 3, we define the GET method to retrieve the PFD for the application(s) by the client, the server responds a whole representation of the resource for the requested application identifier(s) even in the case that the client requests the PFDs when the caching timer expires and the PFD(s) is not changed at the server. It brings a large signalling load if the whole set of the PFD(s) are always returned by considering that the number of the PFD(s) is very large in the deployment.

For push mode, in order to protect the PCEF/TDF from overload, CT3 introduces the notification push for the Push mode in EPS, i.e. the PFDF sends a request with notification flag to the PCEF/TDF and then PCFE/TDF initiates separate pull requests to retrieve the PFDs for different application identifier(s) immediately or within the allowed-delay by considering the load of the PCEF/TDF. But this optimized solution is not defined in 5GS.

CT3#110e decided to implement the solution to resolve the above identified issues.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=880032

[1]		3GPP TS 29.251: "Gw and Gwn reference points for sponsored data connectivity".

[2]		3GPP TS 29.551: "5G System; Packet Flow Description Management Service; Stage 3".

[3]		3GPP TS 29.513: "5G System; Policy and Charging Control signalling flows and QoS parameter mapping; Stage 3".

| 940002 | Restoration of Profiles related to UDR | ReP_UDR | C4 | CP-213070 | Ishikawa, Hiroshi, NTT DOCOMO |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by NTT DOCOMO in CP-220343.

The "Restoration of Profiles related to UDR" (ReP_UDR) enables restoration of profiles between UDR and related NF consumers, to maintain the status up to date as much as possible even in case data stored in UDR is lost or corrupted.

Data stored in UDR is automatically updated in normal operation. Some data is stored in volatile storage units due to its nature, such as location of UE, subscription data, policy data, and loss or corruption of storage containing such data could negatively impact the service to subscribers. Replication of volatile storage units and periodic back-up of data are used to avoid loss or corruption, however further procedures are required in case the integrity of data in UDR cannot be ensured, especially when relying on replicated data.

Following enhancements are specified to fulfil these required procedures:

-	Preparation of profile in NF consumers

-	Path and contents of notification sent from UDR to NF consumers when loss or corruption at UDR is detected

-	Procedure for synchronization initiated by the NF consumer upon receiving notification of loss or corruption

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=940002

| 890070 | PAP/CHAP protocols usage in 5GS | PAP_CHAP | ct | CP-202251 | Mingxue Li, China Telecom |
| --- | --- | --- | --- | --- | --- |
| 890071 | CT1 aspects of PAP/CHAP protocols usage in 5GS | PAP_CHAP | C1 | CP-202251 | Mingxue Li, China Telecom |
| 890072 | CT3 aspects of PAP/CHAP protocols usage in 5GS | PAP_CHAP | C3 | CP-210188 | Mingxue Li, China Telecom |

Summary based on the input provided by China Telecom in CP-202251.

This work item enables the support of the (Extended) Protocol Configuration Options [(e)PCO] parameters related to the Password Authentication Protocol (PAP) / Challenge Handshake Authentication Protocol (CHAP) over 5GS.

This is needed in the following cases:

- migration from EPS to 5GS and potential requirements related to legacy deployments for access to corporate networks, e.g. support of PAP/CHAP in DN-AAA server owned by 3rd parties,

- access to corporate networks by early 5GS deployments (e.g. with standalone SMF), e.g. to support PAP/CHAP in DN-AAA server or LNS owned by 3rd parties.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=890070,890071,890072

[1]	TS 29.561: “Interworking between 5G Network and external Data Networks”

[2]	TS 24.501: “Non-Access-Stratum (NAS) protocol for 5G System (5GS)”

[3]	TS 24.526: “User Equipment (UE) policies for 5G System (5GS)”

# 24	Other Release 17 Features

| tbd | Non-Seamless WLAN offload authentication in 5GS | NSWO_5G | C4 | C4-220349 | WIEHE, Ulrich, Nokia |
| --- | --- | --- | --- | --- | --- |
| 910091 | Study on non-seamless WLAN Offload in 5GS using 3GPP credentials | FS_NSWO_5G | S3 | SP-210262 | Nair, Suresh, Nokia |
| 940011 | Security aspects of Non-Seamless WLAN offload in 5GS | NSWO_5G | S3 | SP-211358 | Ranganathan Mavureddi Dhanasekaran, Nokia |

Summary based on the input provided by Nokia in CP-220150.

The work item defines stage 3 protocol enhancements for Non-seamless WLAN offload (NSWO) authentication in 5GS according to stage 2 requirements from TS 33.501 annex S.

NSWO is an optional capability of a UE supporting WLAN radio access. A UE supporting non-seamless WLAN offload may, while connected to WLAN access, route specific IP flows via the WLAN access without traversing the 3GPP core network.

For authentication, 5G NSWO uses EAP-AKA' as specified in RFC 5448.

A new network function, called NSWOF, supports authentication for NSWO in 5GS. The NSWOF interfaces the WLAN access network via SWa and the AUSF via the Nausf service based interface (SBI). The AUSF retrieves NSWO-specific authentication information from the UDM via the Nudm service based interface.

In addition, the USIM and/or ME can be configured to use 5G NSWO.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=910091,940011

As well as the following CRs:

[1]	CR 29.503 #0790, C4-220141, NSWO authentication

[2]	CR 29.509 #0143r1, C4-221545, NSWO authentication

[3]	CR 29.510 #0669r1, C4-221469, NSWO NF

[4]	CR 29.273 #0530, C4-221155, Non-Seamless WLAN offload in 5GS

[5]	CR 24.302 #0726r1, C1-222069, Add ANID for 5G NSWO

[6]	CR 24.502 #0198r1, C1-222070, Add support of 5G NSWO

[7]	CR 24.501 #4123, C1-222072, Extension of SNN description for NSWO

[8]	CR 31.102 #0946, C6-220154, 5G NSWO (Non-Seamless WLAN Offload) configuration support in the USIM compromised proposal.

[9]	CR 31.111 #0769r2, C6-220174, Addition of the procedure for updating of 5G NSWO configuration in the USIM

| 910007 | Start of Pause of Charging via User Plane | SPOCUP | C4 | CP-210076 | YONG, YANG, Ericsson, frank |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Ericsson in CP-220211.

The work item defines a User Plane (GTP-U) based solution to start pause of charging for a PDN connection or a PDU session to reduce the Charging Data Record discrepancies between SGW or I/V-SMF and PGW or (h)SMF caused by the control plane signalling latency in existing solution.

PGW/SMF Pause of Charging feature is to limit a mismatch between PGW-C/SMF and SGW/I/V-SMF charging volume and packet counts. To further reduce charging discrepancies due to control plane signalling latency, the work item specifies following enhancements to enable start of pause of charging via user plane path:

-		a new feature "Notify Start Pause of Charging via user plane feature" (NSPOC) is introduced in PFCP to enable CP function ((SGW-C and PGW-C for EPC, I/V-SMF and (H-)SMF for 5GC) to instruct UP function (SGW-U and PGW-U for EPC, I/V-UPF and PSA UPF), to trigger pause of charging at the upstream GTP-U entity via sending one or more GTP-U Tunnel Status messages, so that the upstream GTP-U entity can stop usage measurement for those URRs applicable for charging; and

-		a new mechanism to stop/resume the usage measurement in PFCP when NSPOC is supported; and

-	  a new GTP-U message "Tunnel Status" is introduced in GTP-U protocol to be used to notify the upstream GTP-U entity to start pause of charging; and

-		the support of NSPOC feature is exchanged via control plane signalling between PGW-C/SMF and SGW/I/V-SMF respectively.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=910007

| 911037 | Reliable Data Service Serialization Indication | RDSSI |  | SP-190446 | Starsinic, Michael, Convida Wireless LLC |
| --- | --- | --- | --- | --- | --- |
| 840017 | Stage 2 of RDSSI | RDSSI | S2 | SP-190446 | Starsinic, Michael, Convida Wireless LLC |
| 900005 | CT aspects of RDSSI | RDSSI | C1 | CP-203234 | Vivek Gupta, Intel |
| 930002 | IMS voice service support and network usability guarantee for UE s E-UTRA capability disabled scenario in SA 5GS | ING_5GS | ct | CP-212231 | Mingxue Li, China Telecom |
| 930047 | CT1 aspects of ING_5GS | ING_5GS | C1 | CP-212231 | Mingxue Li, China Telecom |
| 930046 | CT6 aspects of ING_5GS | ING_5GS | C6 | CP-212231 | Mingxue Li, China Telecom |
| 930031 | User Consent for 3GPP services | FS_UC3S | S3 | SP-200885 | Rong Wu, Huawei Technologies |
| 890037 | Study on User Consent for 3GPP services | FS_UC3S | S3 | SP-200885 | Rong Wu, Huawei Technologies |
| 930006 | Security aspects on User Consent for 3GPP services | UC3S_SEC | S3 | SP-210836 | Rong Wu, Huawei Technologies |
| 920060 | NPN support of PWS | NPN_PWS | S1 | SP-210585 | Francesco Pica, Qualcomm |
| 910024 | Enhancements of 3GPP profiles for cryptographic algorithms and security protocols | eCryptPr | S3 | SP-210107 | Pinar Comak, Ericsson |
| 920027 | AKMA TLS protocol profiles | AKMA_TLS | S3 | SP-210424 | Escott, Adrian, Qualcomm |
| 910025 | User Plane Integrity Protection for LTE | UPIP_SEC_LTE | S3 | SP-210105 | Evans, Tim, Vodafone |
| 900019 | Adapting BEST for use in 5G networks | BEST_5G | S3 | SP-201020 | Keesmaat, Iko, KPN |
| 840048 | Enhanced Relays for Energy eFficiency and Extensive Coverage | REFEC |  | SP-190307 | Norp, Toon, KPN |
| 810018 | Study on REFEC | FS_REFEC | S1 | SP-180785 | Jose Luis Almodovar Chico, KPN |
| 840034 | Stage 1 of REFEC | REFEC | S1 | SP-190307 | Norp, Toon, KPN |
| 840044 | Network Controlled Interactive Service | NCIS |  | SP-190303 | Ning YANG, oppo |
| 800015 | Study on NCIS | FS_NCIS | S1 | SP-180341 | YANG Ning, Oppo |
| 840030 | Stage 1 of NCIS | NCIS | S1 | SP-191039 | Ning YANG, oppo |
| 840019 | 5G System Enhancement for Advanced Interactive Services | 5G_AIS | S2 | SP-190564 | Lei Yixue, Tencent |
| 890009 | Points for 8K VR 360 Video over 5G | 8K_VR_5G | S4 | SP-200667 | Ozgur Oyman, Company: Intel |
| 880043 | Multi-Device and multi-identity Enhancements (MuDE) | MuDE |  | SP-190823 | Adrian.buckley@vivo.com |
| 850041 | Stage 1 of MuDE | MuDE | S1 | SP-190823 | Adrian.buckley@vivo.com |
| 880018 | Stage 3 of MuDE | MuDe | C1 | CP-201162 | Adrian Buckley |
| 920055 | Best Practice of Packet Forwarding Control Protocol (PFCP) | BEPoP | ct | CP-212024 | Song Yue, China Mobile |
| 920056 | CT3 aspects of BEPoP | BEPoP | C3 | CP-212024 | Song Yue, China Mobile |
| 880014 | CT4 aspects of BEPoP | BEPoP | C4 | CP-212024 | Song Yue, China Mobile |
| 880029 | Enhancement of Handover Optimization | E_HOO | S5 | SP-200466 | Elmdahl, Per, Ericsson |
| 880012 | Extension for headset interface tests of UE | HInT | S4 | SP-200398 | Jan Reimes, HEAD acoustics GmbH, |
| 840038 | MC services support on IOPS mode of operation | MCIOPS | S6 | SP-190944 | Camilo.solano@ericsson.com |
| 840016 | Supporting Flexible Local Area Data Network | FLADN | S2 | SP-190563 | LG Electronics, Hyunsook Kim |
| 830017 | Verification-modified Calling Name Display | VMOD_DISPLAY | S1 | SP-190085 | Hala Mowafy, Ericsson |
| 850038 | Enhanced Calling Name Service Analytics Interworking | eCNAM_An | S1 | SP-190940 | Hala.Mowafy@ericsson.com |
| 910090 | Integration of Generic Bootstrapping Architecture (GBA) into 5GC | GBA_5G |  | SP-190714 | Vlasios Tsiatsis, Ericsson |
| 850023 | Security aspects of Integration of GBA into 5GC | GBA_5G | S3 | SP-190714 | Vlasios Tsiatsis, Ericsson |
| 910004 | CT aspects of Integration of GBA into SBA | GBA_5G | C4 | CP-210283 | de Gregorio, Jesús (Ericsson) |
| 910047 | Integration of Generic Bootstrapping Architecture (GBA) into 5GC | GBA_5G | S3 | SP-190714 | Vlasios Tsiatsis, Ericsson |
| 770024 | EVS Codec Extension for Immersive Voice and Audio Services | IVAS_Codec | S4 | SP-170611 | Bin Wang, Huawei Technologies Co Ltd |
| 860008 | IMS emergency support for Stand-alone Non-Public Network (SNPN) | IESNPN | S1 | SP-191038 | Wong, Curt; |
| 860016 | Assurance Specification for IMS | SCAS_IMS | S3 | SP-191128 | Bo Zhang, Huawei Technologies |
| 870020 | Security Assurance Specification for 5G (eSCAS_5G) | eSCAS_5G | S3 | SP-200149 | Rong Wu, Huawei Technologies Co. |
| 890013 | eSCAS_5G for Network Slice-Specific Authentication and Authorization Function (NSSAAF) | SCAS_5G_NSSAAF | S3 | SP-200720 | Rong Wu, Huawei Technologies Co., Ltd. |
| 870017 | eSCAS_5G for Non-3GPP InterWorking Function | SCAS_5G_N3IWF | S3 | SP-200146 | Feng Gao, China Unicom |
| 870018 | eSCAS_5G for 5G NWDAF | SCAS_5G_NWDAF | S3 | SP-200147 | QI Minpeng, China Mobile |
| 870019 | eSCAS_5G for Service Communication Proxy | SCAS_5G_SECOP | S3 | SP-200148 | Wei Lu, Nokia |
| 880003 | eSCAS_5Gfor Inter PLMN UP Security | SCAS_5G_IPUPS | S3 | SP-200348 | Jin PENG, ZTE Corporation |
| 860025 | Lawful Interception Rel-17 | LI17 | S3 | SP-190983 | Alex Leadbeater, BT |
| 850047 | (Small) Technical Enhancements and Improvements for Rel-17 | TEI17 |  |  |  |
| 870003 | IMS Optimization for HSS Group ID in an SBA environment | TEI17_IMSGID | S2 | SP-200452 | Foti, George, Ericsson |
| 930036 | System enhancement for Redundant PDU Session | TEI17_SE_RPS |  | SP-200448 | Devaki Chandramouli |
| 880017 | Stage 2 for TEI17_SE_RPS | TEI17_SE_RPS | S2 | SP-200448 | Devaki Chandramouli |
| 930001 | CT aspects of TEI17_SE_RPS | TEI17_SE_RPS | ct | CP-212099 | WON, Sung Hwan, Nokia |
| 930037 | CT1 aspects of TEI17_SE_RPS | TEI17_SE_RPS | C1 | CP-212099 | WON, Sung Hwan, Nokia |
| 930038 | CT3 aspects of TEI17_SE_RPS | TEI17_SE_RPS | C3 | CP-212099 | WON, Sung Hwan, Nokia |
| 930039 | CT4 aspects of TEI17_SE_RPS | TEI17_SE_RPS | C4 | CP-212099 | WON, Sung Hwan, Nokia |
| 910078 | Support for Signed Attestation for Priority and Emergency Sessions | TEI17_SAPES |  | SP-200453 | Foti, George, Ericsson |
| 870004 | Stage 2 for TEI17_SAPES | TEI17_SAPES | S2 | SP-200453 | Foti, George, Ericsson |
| 910020 | CT aspects on TEI17_SAPES | TEI17_SAPES | ct | CP-210272 | Axell, Jörgen, Ericsson |
| 910079 | CT1 aspects on TEI17_SAPES | TEI17_SAPES | C1 | CP-210272 | Axell, Jörgen, Ericsson |
| 910080 | CT3 aspects on TEI17_SAPES | TEI17_SAPES | C3 | CP-210272 | Axell, Jörgen, Ericsson |
| 910081 | Dynamically Changing AM Policies in the 5GC | TEI17_DCAMP |  | SP-200446 | Pancorbo, Belen (Ericsson) |
| 870005 | Stage 2 for TEI17_DCAMP | TEI17_DCAMP | S2 | SP-200446 | Pancorbo, Belen (Ericsson) |
| 910009 | CT aspects on TEI17_DCAMP | TEI17_DCAMP | ct | CP-212163 | Garcia Azorero, Fuencisla. Ericsson |
| 910082 | CT3 aspects on TEI17_DCAMP | TEI17_DCAMP | C3 | CP-212163 | Garcia Azorero, Fuencisla. Ericsson |
| 910083 | CT4 aspects on TEI17_DCAMP | TEI17_DCAMP | C4 | CP-212163 | Garcia Azorero, Fuencisla. Ericsson |
| 870006 | IP address pool information from UDM | TEI17_IPU | S2 | SP-200454 | Stefan Rommer |
| 910084 | Dynamic management of group-based event monitoring | TEI17_GEM |  | SP-200455 | Qian XB Chen, Ericsson |
| 870007 | Stage 2 for TEI17_GEM | TEI17_GEM | S2 | SP-200455 | Qian XB Chen, Ericsson |
| 910010 | CT aspects on TEI17_GEM | TEI17_GEM | ct | CP-212165 | Maria Tianmei, Liang, Ericsson |
| 910085 | CT3 aspects on TEI17_GEM | TEI17_GEM | C3 | CP-212165 | Maria Tianmei, Liang, Ericsson |
| 910086 | CT4 aspects on TEI17_GEM | TEI17_GEM | C4 | CP-212165 | Maria Tianmei, Liang, Ericsson |
| 920041 | Support of different slices over different Non 3GPP access | TEI17_N3SLICE |  | SP-200456 | Laurent Thiebaut, Nokia |
| 870008 | Stage 2 for TEI17_N3SLICE | TEI17_N3SLICE | S2 | SP-200456 | Laurent Thiebaut, Nokia |
| 930099 | CT1 aspects of TEI17_N3SLICE | TEI17_N3SLICE | C1 | CP-211089 | LANDAIS, Bruno, Nokia |
| 920001 | CT4 aspects of TEI17_N3SLICE | TEI17_N3SLICE | C4 | CP-211089 | LANDAIS, Bruno, Nokia |
| 910087 | N7/N40 Interfaces Enhancements to Support GERAN and UTRAN | TEI17_NIESGU |  | SP-200457 | Aihua, Li, China Mobile |
| 870009 | Stage 2 for TEI17_NIESGU | TEI17_NIESGU | S2 | SP-200457 | Aihua, Li, China Mobile |
| 890019 | SA5 aspects of N40 Interface Enhancements to Support GERAN and UTRAN | TEI17_NIESGU | S5 | SP-200854 | Dong, Jia, China Mobile |
| 910011 | CT3 aspects of N7 Interfaces Enhancements to Support GERAN and UTRAN | TEI17_NIESGU | C3 | CP-211194 | Huang Zhenning (China Mobile) |
| 920082 | CT4 aspects of N7 Interfaces Enhancements to Support GERAN and UTRAN | TEI17_NIESGU | C4 | CP-211194 | Huang Zhenning (China Mobile) |

Summary based on the input provided by XXX in xP-22xxxx.

# 25	Management-only Features

This section presents all the Features with no corresponding work in other group than the Operations, administration and management (OAM) group, i.e. SA5. For instance, OAM for "Industrial IoT" is reported in the section on "Industrial IoT" and not here.

| 870022 | Enhancements on EE for 5G networks | EE5GPLUS | S5 | SP-200188 | Jean-Michel CORNILY, ORANGE |
| --- | --- | --- | --- | --- | --- |
| 870024 | Enhancement on Management Aspects of 5G Service-Level Agreement | EMA5SLA | S5 | SP-210860 | SHI, Xiaonan, CMCC |
| 870025 | Management of MDT enhancement in 5G | e_5GMDT | S5 | SP-200191 | Ayani, Zhulia, Ericsson |
| 870026 | Additional NRM features | adNRM | S5 | SP-200192 | Jing Ping, Nokia |
| 870027 | Enhancement of QoE Measurement Collection | eQoE | S5 | SP-200193 | Petersen, Robert, Ericsson |
| 860061 | Study on NR QoE (Quality of Experience) management and optimizations for diverse services | FS_NR_QoE | R3 | RP-193256 | China Unicom |
| 870030 | Enhanced Closed loop SLS Assurance | eCOSLA | S5 | SP-200196 | Groenendijk, Jan, Ericsson |
| 840028 | IMS Charging in 5G System Architecture | 5GSIMSCH | S5 | SP-190367 | Jahangir, Zeeshan, T-Mobile USA |
| 810027 | Intent driven management service for mobile network | IDMS_MN | S5 | SP-180899 | Zou Lan, Huawei |
| 820006 | Study on User Plane Integrity Protection | FS_UP_IP_Sec | S3 | SP-181035 | Evans, Tim, Vodafone |
| 890012 | Enhancements to User Plane Integrity Protection Support in 5GS | eUPIP_SEC | S3 | SP-200719 | Anand Palanigounder, Qualcomm |
| 820035 | Discovery of management services in 5G | 5GDMS | S5 | SP-200770 | Brendan Hassett, Huawei Technologies Sweden AB |
| 860024 | Network policy managementfor 5G mobile networks | NPM | S5 | SP-191211 | China Mobile Shasha Guo |
| 850028 | Study on enhancement of Management Data Analytics Service | FS_eMDAS | S5 | SP-190930 | Yao, Yizhi, Intel |
| 910027 | Enhancements of Management Data Analytics Service | eMDAS | S5 | SP-210132 | Yao, Yizhi, Intel |
| 880028 | Management data collection control and discovery | MADCOL | S5 | SP-200465 | Pollakowski, Olaf, Nokia |
| 880026 | Management of the enhanced tenant concept | eMEMTANE | S5 | SP-200463 | Zhu, Lei, Huawei |
| 880025 | Enhancements of 5G performance measurements and KPIs | ePM_KPI_5G | S5 | SP-200462 | Yizhi Yao, Intel |
| 930032 | Improved support for NSA in the service-based management architecture | NSA_SBMA | S5 | SP-211121 | Lan Zou, Huawei |
| 910031 | Study on Enhancement of service-based management architecture | FS_eSBMA | S5 | SP-210136 | Lan Zou, Huawei |
| 930009 | Improved support for NSA in the service-based management architecture | NSA_SBMA | S5 | SP-210858 | Lan Zou, Huawei |
| 930033 | Access control for management service | MSAC | S5 | SP-210859 | Jing Ping, Nokia |
| 890016 | Study on access control for management service | FS_MNSAC | S5 | SP-200853 | Jing Ping, Nokia |
| 930010 | Access control for management service | MSAC | S5 | SP-210859 | Jing Ping, Nokia |
| 910030 | File Management | FIMA | S5 | SP-210135 | Pollakowski, Olaf, Nokia |
| 900021 | Management Aspects of 5G Network Sharing | MANS | S5 | SP-201080 | Mingrui Sun, China Unicom |
| 920018 | Study on Management Aspects of 5G Network Sharing | FS_MANS | S5 | SP-210387 | Mingrui Sun, China Unicom |

# 26	Features without summary

The Features listed below are not summarised in this document because their output is not significant enough. It can e.g. be some minor protocol enhancements.

The corresponding CR(s), if any, can be found by replacing [UID] by the actual UID in the link below, selecting "TSG Status = Approved" in the page:

https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=[UID]

The UID is the left-most number in the table below.

| 880020 | Protocol enhancements for Mission Critical Services | MCProtoc17 | C1 | CP-202193 | AXELL, Jörgen |
| --- | --- | --- | --- | --- | --- |
| 880019 | Stage-3 5GS NAS protocol development 17 | 5GProtoc17 | C1 | CP-201163 | Sedlacek, Ivo, Ericsson |
| 880044 | Stage-3 5GS NAS protocol development 17 general aspects | 5GProtoc17 | C1 | CP-201163 | Sedlacek, Ivo, Ericsson |
| 880045 | Stage-3 5GS NAS protocol development 17 non-IETF aspects | 5GProtoc17-non3GPP | C1 | CP-201163 | Sedlacek, Ivo, Ericsson |
| 880021 | Stage-3 SAE Protocol Development | SAES17 | C1 | CP-201165 | Aghili, Behrouz, InterDigital Communications |
| 880046 | Stage-3 SAE Protocol Development general aspects | SAES17 | C1 | CP-201165 | Aghili, Behrouz, InterDigital Communications |
| 880047 | Stage-3 SAE Protocol Development CSFB aspects | SAES17-CSFB | C1 | CP-201165 | Aghili, Behrouz, InterDigital Communications |
| 880048 | Stage-3 SAE Protocol Development non 3GPP aspects | SAES17-non3GPP | C1 | CP-201165 | Aghili, Behrouz, InterDigital Communications |
| 911034 | IMS Stage-3 IETF Protocol Alignment | IMSProtoc17 | ct | CP-201167 | Gkatzikis, Lazaros(Nokia) |
| 880023 | CT1 aspects of IMS Stage-3 IETF Protocol Alignment | IMSProtoc17 | C1 | CP-201167 | Gkatzikis, Lazaros(Nokia) |
| 911035 | CT3 aspects of IMS Stage-3 IETF Protocol Alignment | IMSProtoc17 | C3 | CP-201167 | Gkatzikis, Lazaros(Nokia) |
| 880013 | Service Based Interface Protocol Improvements Release 17 | SBIProtoc17 |  | CP-211088 | Song Yue, China Mobile |
| 880053 | CT3 aspects of SBIProtoc17 | SBIProtoc17 | C3 | CP-211088 | Song Yue, China Mobile |
| 880054 | CT4 aspects of SBIProtoc17 | SBIProtoc17 | C4 | CP-211088 | Song Yue, China Mobile |

Annex A:
List of expected contributions

Summary needed for highlighted items. Summary to be produced by the Rapporteur as highlighted in the table below.

| UID | Name | A c r o n y m | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 0 | Rel-17 on Use of 5G by different sectors ("verticals") | - |  | - | - |
| 840050 | Enhancements for cyber-physical control applications in vertical domains | e C A V |  | SP-190310 | Bahr, Michael, Siemens AG |
| 830020 | Study on eCAV | F S _ e C A V | S1 | SP-190092 | Michael Bahr, Siemens AG |
| 840041 | Stage 1 of eCAV | e C A V | S1 | SP-191043 | Bahr, Michael, Siemens AG |
| 840024 | Study on enhanced support of Non-Public Networks | F S _ e N P N | S2 | SP-200094 | Peter Hedman |
| 910065 | Enhanced support of Non-Public Networks | e N P N |  | SP-200980 | Hedman, Peter, Ericsson |
| 900015 | Stage 2 for eNPN | e N P N | S2 | SP-200980 | Hedman, Peter, Ericsson |
| 910016 | CT aspects of eNPN | e N P N | ct | CP-212103 | Sedlacek, Ivo, Ericsson |
| 910066 | CT1 aspects of eNPN | e N P N | C1 | CP-212103 | Sedlacek, Ivo, Ericsson |
| 910067 | CT3 aspects of eNPN | e N P N | C3 | CP-212103 | Sedlacek, Ivo, Ericsson |
| 910068 | CT4 aspects of eNPN | e N P N | C4 | CP-212103 | Sedlacek, Ivo, Ericsson |
| 920025 | Security Aspects of eNPN | e N P N | S3 | SP-210422 | Jost, Christine, Ericsson |
| 880008 | Study on enhanced security support for Non-Public Networks | F S _ e N P N _ S E C | S3 | SP-200353 | Normann, Henrik, Ericsson |
| 840045 | Audio-Visual Service Production | A V P R O D |  | SP-190304 | Wagdin, Ian, BBC |
| 800014 | Study on AVPROD | F S _ A V P R O D | S1 | SP-181015 | Roland Beutler, EBU |
| 840031 | Stage 1 of AVPROD | A V P R O D | S1 | SP-191041 | Wagdin, Ian, BBC |
| 910001 | Study on Media Production over 5G NPN | F S _ N P N 4 A V P r o d | S4 | SP-210241 | Lohmar, Thorsten, Ericsson |
| 820025 | Study on application layer support for Factories of the Future in 5G network | F S _ F F A P P | S6 | SP-200836 | Shao Weixiang, ZTE Corporation |
| 840025 | Study on enhancement of support for 5G LAN-type service | F S _ 5 G L A N _ e n h | S2 | SP-190626 | Runze Zhou, Huawei |
| 900022 | Study on Charging Aspect of 5G LAN-type Services | F S _ 5 G L A N _ C H | S5 | SP-201081 | CHEN SHAN, Huawei |
| 840042 | Complete Gap Analysis for Railways Mobile Communication System | M O N A S T E R Y E N D | S1 | SP-190312 | Gach, Guillaume, UIC |
| 890035 | Enhancements to Application Architecture for the Mobile Communication System for Railways Phase 2 | e M O N A S T E R Y 2 |  | SP-191104 | Oettl, Martin, Nokia |
| 840037 | Stage 2 of eMONASTERY2 | e M O N A S T E R Y 2 | S6 | SP-191104 | Oettl, Martin, Nokia |
| 890063 | Stage 3 of eMONASTERY2 | e M O N A S T E R Y 2 | C1 | CP-202256 | Gkatzikis, Lazaros, Nokia(Nokia) |
| 840043 | Study on Future Railway Mobile Communication System3 | F S _ F R M C S 3 | S1 | SP-200578 | Gach, Guillaume, UIC, |
| 850046 | Asset Tracking for 5G | A T R A C |  | SP-190816 | Thierry Berisot, NOVAMINT |
| 810017 | Study on ATRAC | F S _ 5 G _ A T R A C | S1 | SP-180922 | Thierry Berisot, NOVAMINT |
| 850037 | Stage 1 of ATRAC | A T R A C | S1 | SP-190931 | Thierry Berisot, NOVAMINT |
| 910052 | Enhancement to the 5GC LoCation Services-Phase 2 | 5 G _ e L C S _ p h 2 |  | SP-200082 | Ming Ai, CATT |
| 870001 | Stage 2 of 5G_eLCS_ph2 | 5 G _ e L C S _ p h 2 | S2 | SP-200082 | Ming Ai, CATT |
| 910006 | CT aspects of 5G_eLCS_ph2 | 5 G _ e L C S _ p h 2 | ct | CP-211090 | Chenxi Bao, CATT |
| 910053 | CT1 aspects of 5G_eLCS_ph2 | 5 G _ e L C S _ p h 2 | C1 | CP-211090 | Chenxi Bao, CATT |
| 910054 | CT3 aspects of 5G_eLCS_ph2 | 5 G _ e L C S _ p h 2 | C3 | CP-211090 | Chenxi Bao, CATT |
| 910055 | CT4 aspects of 5G_eLCS_ph2 | 5 G _ e L C S _ p h 2 | C4 | CP-211090 | Chenxi Bao, CATT |
| 900060 | RAN aspects of NR positioning enhancements | N R _ p o s _ e n h |  | RP-210903 | Intel |
| 860034 | Study on NR positioning enhancements | F S _ N R _ p o s _ e n h | R1 | RP-202094 | CATT |
| 900160 | Core part: NR positioning enhancements | N R _ p o s _ e n h - C o r e | R1 | RP-210903 | Intel |
| 900260 | Perf. part: NR positioning enhancements | N R _ p o s _ e n h - P e r f | R4 | RP-210903 | Intel |
| 840047 | Communication Service Requirements for Critical Medical Applications | C M E D |  | SP-190306 | Lagrange, Mathieu, b<>com |
| 810016 | Study on CMED | F S _ C M E D | S1 | SP-180783 | Mathieu Lagrange, b<>com |
| 840033 | Stage 1 of CMED | C M E D | S1 | SP-190306 | Lagrange, Mathieu, b<>com |
| 0 | Rel-17 drone/UAS/UAV/EAV | - |  | - | - |
| 840083 | 5G Enhancement for UAVs | E A V |  | SP-190308 | Qun Wei, China Unicom |
| 810019 | Study on EAV | F S _ E A V | S1 | SP-180909 | Qun Wei, China Unicom |
| 840039 | Stage 1 of EAV | E A V | S1 | SP-190308 | Qun Wei, China Unicom |
| 820011 | Study on supporting Uncrewed Aerial Systems Connectivity, Identification, and Tracking | F S _ I D _ U A S _ S A 2 | S2 | SP-200097 | Qualcomm, Stefano Faccin |
| 820026 | Study on application layer support for Uncrewed Aerial System (UAS) | F S _ U A S A P P | S6 | SP-200111 | Niranth Amogh, Huawei |
| 920045 | Application layer support for Uncrewed Aerial System (UAS) | U A S A P P |  | SP-200988 | Niranth Amogh, Huawei Telecommunications India |
| 900025 | Stage 2 of Application layer support for Uncrewed Aerial System (UAS) | U A S A P P | S6 | SP-200988 | Niranth Amogh, Huawei Telecommunications India |
| 920004 | CT1 Aspects of Application Layer Support for Uncrewed Aerial Systems (UAS) | U A S A P P | C1 | CP-211111 | Lin Shu, Huawei, |
| 920046 | CT3 Aspects of Application Layer Support for Uncrewed Aerial Systems (UAS) | U A S A P P | C3 | CP-211111 | Lin Shu, Huawei |
| 880007 | Study on security aspects of Uncrewed Aerial Systems (UAS) | F S _ U A S _ S E C | S3 | SP-200352 | Adrian Escott, Qualcomm |
| 810049 | Remote Identification of Uncrewed Aerial Systems | I D _ U A S |  | SP-180771 | Eddy HALL, Qualcomm |
| 810013 | Stage 1 of Remote Identification of Uncrewed Aerial Systems | I D _ U A S | S1 | SP-180771 | Eddy HALL, Qualcomm |
| 900014 | (Stage 2 of) Support of Uncrewed Aerial Systems Connectivity, Identification, and Tracking | I D _ U A S | S2 | SP-200979 | Faccin, Stefano, Qualcomm |
| 910017 | CT aspects for ID_UAS | I D _ U A S | ct | CP-211112 | Sunghoon Kim, Qualcomm |
| 910069 | CT1 aspects for ID_UAS | I D _ U A S | C1 | CP-211112 | Sunghoon Kim, Qualcomm |
| 910070 | CT3 aspects for ID_UAS | I D _ U A S | C3 | CP-211112 | Sunghoon Kim, Qualcomm |
| 910071 | CT4 aspects for ID_UAS | I D _ U A S | C4 | CP-211112 | Sunghoon Kim, Qualcomm |
| 920028 | Security aspects of Uncrewed Aerial Systems | I D _ U A S | S3 | SP-210425 | Escott, Adrian, Qualcomm |
| 0 | Rel-17 Satellite/Non-Terrestrial (NTN) related | - |  | - | - |
| 890034 | Integration of satellite components in the 5G architecture | 5 G S A T _ A R C H |  | SP-191335 | Cyril MICHEL, thales |
| 800048 | Stage 1 of 5GSAT | 5 G S A T | S1 | SP-180326 | MICHEL, Cyril, Thales |
| 800026 | Study on architecture aspects for using satellite access in 5G | F S _ 5 G S A T _ A R C H | S2 | SP-181253 | MICHEL, Cyril |
| 860005 | (Stage 2 of) Integration of satellite components in the 5G architecture | 5 G S A T _ A R C H | S2 | SP-191335 | Cyril MICHEL, thales |
| 911030 | CT aspects of 5GC architecture for satellite networks | 5 G S A T _ A R C H - C T | ct | CP-210149 | Catovic, Amer, Qualcomm |
| 890005 | CT1 aspects of 5GC architecture for satellite networks | 5 G S A T _ A R C H - C T | C1 | CP-210149 | Catovic, Amer, Qualcomm |
| 920057 | CT3 aspects of 5GC architecture for satellite networks | 5 G S A T _ A R C H - C T | C3 | CP-210149 | Catovic, Amer, Qualcomm |
| 911031 | CT4 aspects of 5GC architecture for satellite networks | 5 G S A T _ A R C H - C T | C4 | CP-210149 | Catovic, Amer, Qualcomm |
| 930044 | CT6 aspects of 5GC architecture for satellite networks | 5 G S A T _ A R C H - C T | C6 | CP-210149 | Catovic, Amer, Qualcomm |
| 860046 | Solutions for NR to support non-terrestrial networks (NTN) | N R _ N T N _ s o l u t i o n s |  | RP-210908 | Thales |
| 860146 | Core part: Solutions for NR for NTN | N R _ N T N _ s o l u t i o n s - C o r e | R2 | RP-210908 | Thales |
| 860246 | Perf. Part: Solutions for NR for NTN | N R _ N T N _ s o l u t i o n s - P e r f | R4 | RP-210908 | Thales |
| 920069 | NB-IoT/eMTC support for Non-Terrestrial Networks | L T E _ N B I O T _ e M T C _ N T N | R1 | RP-211601 | MediaTek |
| 920169 | Core part: NB-IoT/eMTC support for Non-Terrestrial Networks | L T E _ N B I O T _ e M T C _ N T N - C o r e | R1 | RP-211601 | MediaTek |
| 930019 | Architecture support for NB-IoT/eMTC Non-Terrestrial Networks in EPS | I o T _ S A T _ A R C H _ E P S | S2 | SP-211124 | SEBIRE, Guillaume, MediaTek Inc. |
| 830025 | Study on management and orchestration aspects with integrated satellite components in a 5G network | F S _ 5 G S A T _ M O | S5 | SP-190138 | Floris Drijver, TNO |
| 860033 | Study on NB-IoT/eMTC support for NTN | F S _ L T E _ N B I O T _ e M T C _ N T N | R1 | RP-202689 | MediaTek |
| 0 | Rel-17 IoT related | - |  | - | - |
| 850012 | Study on enhanced support of Industrial IoT | F S _ I I o T | S2 | SP-200298 | Nokia, Devaki Chandramouli, |
| 880010 | Study on security for enhanced support of Industrial IoT | F S _ I I o T _ S E C | S3 | SP-200355 | Nokia, Anja Jerichow |
| 920024 | Security for enhanced support of Industrial IoT | I I o T _ S E C | S3 | SP-210421 | Nokia, Anja Jerichow |
| 910059 | Support of Enhanced Industrial IIoT | I I o T |  | SP-200973 | Devaki Chandramouli, Nokia |
| 900008 | Stage 2 for IIoT | I I o T | S2 | SP-200973 | Devaki Chandramouli, Nokia |
| 910014 | CT aspects of support of enhanced Industrial IoT | I I o T | ct | CP-212100 | Won, Sung Hwan, Nokia |
| 910060 | CT1 aspects of support of enhanced Industrial IoT | I I o T | C1 | CP-212100 | Won, Sung Hwan, Nokia |
| 910061 | CT3 aspects of support of enhanced Industrial IoT | I I o T | C3 | CP-212100 | Won, Sung Hwan, Nokia |
| 910062 | CT4 aspects of support of enhanced Industrial IoT | I I o T | C4 | CP-212100 | Won, Sung Hwan, Nokia |
| 860044 | Additional enhancements for NB-IoT and LTE-MTC | N B _ I O T e n h 4 _ L T E _ e M T C 6 |  | RP-211340 | Huawei |
| 860144 | Core part: Additional enhancements for NB-IoT and LTE-MTC | N B _ I O T e n h 4 _ L T E _ e M T C 6 - C o r e | R1 | RP-211340 | Huawei |
| 860244 | Perf. part: Additional enhancements for NB-IoT and LTE-MTC | N B _ I O T e n h 4 _ L T E _ e M T C 6 - P e r f | R4 | RP-211340 | Huawei |
| 860051 | NR small data transmissions in INACTIVE state | N R _ S m a l l D a t a _ I N A C T I V E |  | RP-212594 | ZTE |
| 860151 | Core part: NR small data transmissions in INACTIVE state | N R _ S m a l l D a t a _ I N A C T I V E - C o r e | R2 | RP-212594 | ZTE |
| 860251 | Perf. part: NR small data transmissions in INACTIVE state | N R _ S m a l l D a t a _ I N A C T I V E - P e r f | R4 | RP-212594 | ZTE |
| 860045 | Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR | N R _ I I O T _ U R L L C _ e n h |  | RP-210854 | Nokia |
| 860145 | Core part: Enhanced IoT and URLLC support for NR | N R _ I I O T _ U R L L C _ e n h - C o r e | R2 | RP-210854 | Nokia |
| 860245 | Perf. Part: Enhanced IoT and URLLC support for NR | N R _ I I O T _ U R L L C _ e n h - P e r f | R4 | RP-210854 | Nokia |
| 0 | Rel-17 V2V/D2D/Sidelink related | - |  | - | - |
| 860042 | NR Sidelink enhancement | N R _ S L _ e n h |  | RP-202846 | LG Electronics |
| 860142 | Core part: NR Sidelink enhancement | N R _ S L _ e n h - C o r e | R1 | RP-202846 | LG Electronics |
| 860242 | Perf. part: NR Sidelink enhancement | N R _ S L _ e n h - P e r f | R4 | RP-202846 | LG Electronics |
| 860038 | Study on NR Sidelink relay | F S _ N R _ S L _ r e l a y | R2 | RP-202208 | OPPO |
| 840035 | Study on enhancements to application layer support for V2X services | F S _ e V 2 X A P P | S6 | SP-200110 | Niranth Amogh, Huawei Telecommunications India |
| 910075 | Enhanced application layer support for V2X services | e V 2 X A P P |  | SP-200831 | Niranth Amogh, Huawei Telecommunications India |
| 890036 | Stage 2 of eV2XAPP | e V 2 X A P P | S6 | SP-200831 | Niranth Amogh, Huawei Telecommunications India |
| 910019 | CT aspects of eV2XAPP | e V 2 X A P P | ct | CP-211109 | Herrero Veron, Christian (Huawei) |
| 910076 | CT1 aspects of eV2XAPP | e V 2 X A P P | C1 | CP-211109 | Herrero Veron, Christian (Huawei) |
| 910077 | CT3 aspects of eV2XAPP | e V 2 X A P P | C3 | CP-211109 | Herrero Veron, Christian (Huawei) |
| 910037 | Support of advanced V2X services - Phase 2 | e V 2 X A R C _ P h 2 |  | SP-210090 | LaeYoung Kim, LG Electronics |
| 850013 | Study on V2X services – Phase 2 | F S _ e V 2 X A R C _ P h 2 | S2 | SP-190631 | LaeYoung Kim, LG Electronics |
| 910021 | Stage 2 of eV2XARC_Ph2 | e V 2 X A R C _ P h 2 | S2 | SP-210090 | LaeYoung Kim, LG Electronics |
| 920005 | CT1 aspects of eV2XARC_Ph2 | e V 2 X A R C _ P h 2 | C1 | CP-211116 | Herrero Veron, Christian (Huawei) |
| 920047 | CT3 aspects of eV2XARC_Ph2 | e V 2 X A R C _ P h 2 | C3 | CP-211116 | Herrero Veron, Christian (Huawei) |
| 920048 | CT6 aspects of eV2XARC_Ph2 | e V 2 X A R C _ P h 2 | C6 | CP-211116 | Herrero Veron, Christian (Huawei) |
| 0 | Rel-17 NR enhancements | - |  | - | - |
| 920070 | UE RF requirements for Transparent Tx Diversity (TxD) for NR | N R _ R F _ T x D | R4 | RP-211940 | Qualcomm |
| 920170 | Core part: UE RF requirements for Transparent Tx Diversity (TxD) for NR | N R _ R F _ T x D - C o r e | R4 | RP-211940 | Qualcomm |
| 920270 | Perf. part: UE RF requirements for Transparent Tx Diversity (TxD) for NR | N R _ R F _ T x D - P e r f | R4 | RP-211940 | Qualcomm |
| 860040 | Further enhancements on MIMO for NR | N R _ f e M I M O |  | RP-212535 | Samsung |
| 860140 | Core part: Further enhancements on MIMO for NR | N R _ f e M I M O - C o r e | R1 | RP-212535 | Samsung |
| 860240 | Perf. part: Further enhancements on MIMO for NR | N R _ f e M I M O - P e r f | R4 | RP-212535 | Samsung |
| 860043 | NR Dynamic spectrum sharing (DSS) | N R _ D S S |  | RP-211345 | Ericsson |
| 860143 | Core part: NR DSS | N R _ D S S - C o r e | R1 | RP-211345 | Ericsson |
| 860047 | UE power saving enhancements for NR | N R _ U E _ p o w _ s a v _ e n h |  | RP-212630 | MediaTek |
| 860147 | Core part: UE power saving enhancements for NR | N R _ U E _ p o w _ s a v _ e n h - C o r e | R2 | RP-212630 | MediaTek |
| 860247 | Perf. part: UE power saving enhancements for NR | N R _ U E _ p o w _ s a v _ e n h - P e r f | R4 | RP-212630 | MediaTek |
| 860049 | Further Multi-RAT Dual-Connectivity enhancements | L T E _ N R _ D C _ e n h 2 |  | RP-201040 | Huawei |
| 860149 | Core part: Further Multi-RAT Dual-Connectivity enhancements | L T E _ N R _ D C _ e n h 2 - C o r e | R2 | RP-201040 | Huawei |
| 860249 | Perf. part: Further Multi-RAT Dual-Connectivity enhancements | L T E _ N R _ D C _ e n h 2 - P e r f | R4 | RP-201040 | Huawei |
| 860050 | Enhancements to Integrated Access and Backhaul (IAB) for NR | N R _ I A B _ e n h |  | RP-211548 | Qualcomm |
| 860150 | Core part: Enhancements to IAB for NR | N R _ I A B _ e n h - C o r e | R2 | RP-211548 | Qualcomm |
| 860250 | Perf. part: Enhancements to IAB for NR | N R _ I A B _ e n h - P e r f | R4 | RP-211548 | Qualcomm |
| 830021 | Study on Security for NR_IAB | F S _ N R _ I A B _ S e c | S3 | SP-201016 | Rajavelsamy Rajadurai, Samsung, |
| 860052 | Enhanced eNB(s) architecture evolution for E-UTRAN and NG-RAN | L T E _ N R _ a r c h _ e v o _ e n h |  | RP-211409 | China Unicom |
| 860152 | Core part: LTE_NR_arch_evo_enh | L T E _ N R _ a r c h _ e v o _ e n h - C o r e | R3 | RP-211409 | China Unicom |
| 890055 | Further enhancement on NR demodulation performance | N R _ d e m o d _ e n h 2 |  | RP-212636 | China Telecom |
| 890255 | Perf. part: NR_demod_enh2 | N R _ d e m o d _ e n h 2 - P e r f | R4 | RP-212636 | China Telecom |
| 890056 | Introduction of DL 1024QAM for NR frequency range 1 (FR1) | N R _ D L 1 0 2 4 Q A M _ F R 1 |  | RP-202886 | Ericsson |
| 890156 | Core part: Introduction of DL 1024QAM for NR FR1 | N R _ D L 1 0 2 4 Q A M _ F R 1 - C o r e | R4 | RP-202886 | Ericsson |
| 890256 | Perf. part: Introduction of DL 1024QAM for NR FR1 | N R _ D L 1 0 2 4 Q A M _ F R 1 - P e r f | R4 | RP-202886 | Ericsson |
| 890057 | NR RRM further enhancement | N R _ R R M _ e n h 2 |  | RP-202874 | Apple |
| 890157 | Core part: Further RRM enhancement for NR and MR-DC | N R _ R R M _ e n h 2 - C o r e | R4 | RP-202874 | Apple |
| 890257 | Perf. part: Further RRM enhancement for NR and MR-DC | N R _ R R M _ e n h 2 - P e r f | R4 | RP-202874 | Apple |
| 890058 | Enhanced NR support for high speed train scenario for frequency range 1 (FR1) | N R _ H S T _ F R 1 _ e n h |  | RP-210833 | CMCC |
| 890158 | Core part: Enhanced NR support for high speed train scenario for FR1 | N R _ H S T _ F R 1 _ e n h - C o r e | R4 | RP-210833 | CMCC |
| 890258 | Perf. part: Enhanced NR support for high speed train scenario for FR1 | N R _ H S T _ F R 1 _ e n h - P e r f | R4 | RP-210833 | CMCC |
| 890059 | Further enhancements of NR RF requirements for frequency range 2 (FR2) | N R _ R F _ F R 2 _ r e q _ e n h 2 |  | RP-212092 | Nokia |
| 890159 | Core part: NR_RF_FR2_req_enh2 | N R _ R F _ F R 2 _ r e q _ e n h 2 - C o r e | R4 | RP-212092 | Nokia |
| 890259 | Perf. part: NR_RF_FR2_req_enh2 | N R _ R F _ F R 2 _ r e q _ e n h 2 - P e r f | R4 | RP-212092 | Nokia |
| 890060 | NR support for high speed train scenario in frequency range 2 (FR2) | N R _ H S T _ F R 2 |  | RP-210800 | Samsung |
| 890160 | Core part: NR support for high speed train scenario in FR2 | N R _ H S T _ F R 2 - C o r e | R4 | RP-210800 | Samsung |
| 890260 | Perf. Part: NR support for high speed train scenario in FR2 | N R _ H S T _ F R 2 - P e r f | R4 | RP-210800 | Samsung |
| 890061 | NR measurement gap enhancements | N R _ M G _ e n h |  | RP-211591 | MediaTek |
| 890161 | Core part: NR and MR-DC measurement gap enhancements | N R _ M G _ e n h - C o r e | R4 | RP-211591 | MediaTek |
| 890261 | Perf. part: NR and MR-DC measurement gap enhancements | N R _ M G _ e n h - P e r f | R4 | RP-211591 | MediaTek |
| 890062 | RF requirements enhancement for NR frequency range 1 (FR1) | N R _ R F _ F R 1 _ e n h |  | RP-212527 | Huawei |
| 890162 | Core part: RF requirements enhancement for NR FR1 | N R _ R F _ F R 1 _ e n h - C o r e | R4 | RP-212527 | Huawei |
| 890262 | Perf. part: RF requirements enhancement for NR FR1 | N R _ R F _ F R 1 _ e n h - P e r f | R4 | RP-212527 | Huawei |
| 0 | Rel-17 Self-Organizing (SON)/Autonomous Networks | - |  | - | - |
| 860053 | Enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR and EN-DC | N R _ E N D C _ S O N _ M D T _ e n h |  | RP-193255 | CMCC |
| 860153 | Core part: Enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR and EN-DC | N R _ E N D C _ S O N _ M D T _ e n h - C o r e | R3 | RP-193255 | CMCC |
| 910029 | Plug and connect support for management of Network Functions | P A C M A N | S5 | SP-210260 | Per Elmdahl, Ericsson |
| 870028 | Enhancements of Self-Organizing Networks (SON) for 5G networks | e S O N _ 5 G | S5 | SP-200194 | Joey Chou, Intel |
| 850032 | Study on autonomous network levels | F S _ A N L | S5 | SP-190928 | China Mobile |
| 880027 | Autonomous network levels | A N L | S5 | SP-200464 | Cao, Xi, China Mobile |
| 0 | Rel-17 Multicast-broadcast Services (MBS) and Mission Critical (MC) | - |  | - | - |
| 900038 | Multicast-broadcast services in 5G | 5 M B S |  | SP-201106 | Meng Li, Huawei |
| 830030 | Study on Architectural enhancements for 5G multicast-broadcast services | F S _ 5 M B S | S2 | SP-200690 | Meng Li, Huawei |
| 900009 | Stage 2 for 5MBS | 5 M B S | S2 | SP-201106 | Meng Li, Huawei |
| 920043 | CT1 aspects of the architectural enhancements for 5G multicast-broadcast services | 5 M B S | C1 | CP-212022 | Gulbani, Giorgi, Huawei |
| 920044 | CT3 aspects of the architectural enhancements for 5G multicast-broadcast services | 5 M B S | C3 | CP-212022 | Gulbani, Giorgi, Huawei |
| 910002 | CT4 aspects of the architectural enhancements for 5G multicast-broadcast services | 5 M B S | C4 | CP-212022 | Gulbani, Giorgi, Huawei |
| 920023 | Security Aspects of Enhancements for 5G Multicast-Broadcast Services | 5 M B S | S3 | SP-210420 | Longhua Guo, Huawei Technologies |
| 870014 | Study on Multicast Architecture Enhancements for 5G Media Streaming | F S _ 5 G M S _ M u l t i c a s t | S4 | SP-200238 | Peng Tan, Telus |
| 810055 | Study on location enhancements for mission critical services | F S _ e n h M C L o c | S6 | SP-190725 | Dom Lazara, Motorola Solutions |
| 850040 | Broadcast / Multicast requirements supporting Mission Critical Services in 5G | 5 M B S _ e M C | S1 | SP-190942 | Toobe, Jens, BDBOS |
| 880006 | Study on Security Aspects of Enhancements for 5G Multicast-Broadcast Services | F S _ 5 M B S _ S E C | S3 | SP-200351 | Longhua Guo, Huawei Technologies |
| 850035 | Study on Mission Critical services over 5G multicast-broadcast system | F S _ M C 5 M B S | S6 | SP-190929 | Val Oprescu, AT&T |
| 860048 | NR multicast and broadcast services | N R _ M B S | R2 | RP-193248 | Huawei |
| 860148 | Core part: NR multicast and broadcast services | N R _ M B S - C o r e | R2 | RP-201038 | Huawei |
| 800023 | Study on Mission Critical services support over 5G System | F S _ M C O v e r 5 G S | S6 | SP-200837 | Verweij, Kees, The Police of the Netherlands |
| 920051 | Mission Critical Services over 5GS | M C O v e r 5 G S |  | SP-200833 | Wendler, Ingo, UIC |
| 890027 | Stage 2 of MCOver5GS | M C O v e r 5 G S | S6 | SP-200833 | Wendler, Ingo, UIC |
| 920007 | CT1 aspects of MCOver5GS | M C O v e r 5 G S | C1 | CP-211118 | Gkatzikis, Lazaros, Nokia |
| 890039 | Mission Critical Data | e M C D a t a 3 |  | SP-191106 | Shih, Jerry, AT&T |
| 860007 | Stage 2 of MCData3 | e M C D a t a 3 | S6 | SP-191106 | Shih, Jerry, AT&T |
| 890038 | CT aspects of eMCData3 | e M C D a t a 3 | C1 | CP-201177 |  |
| 880020 | Protocol enhancements for Mission Critical Services | M C P r o t o c 1 7 | C1 | CP-202193 | AXELL, Jörgen |
| 820040 | Mission Critical system migration and interconnection | M C S M I _ C T | C1 | CP-190143 | Dom Lazara, Motorola Solutions |
| 890003 | CT aspects of Enhanced Mission Critical Communication Interworking with Land Mobile Radio Systems | e M C C I _ C T | C1 | eMCCI_CT | Mike Dolan, FirstNet |
| 0 | Rel-17 Edge computing | - |  | - | - |
| 880042 | Architecture for enabling Edge Applications | E D G E A P P |  | SP-200109 | Gupta, Nishant, Samsung |
| 830008 | Study on Application Architecture for enabling Edge Applications | F S _ E D G E A P P | S6 | SP-190065 | Nishant Gupta, Samsung |
| 860006 | Architecture for enabling Edge Applications | E D G E A P P | S6 | SP-200886 | Gupta, Nishant, Samsung |
| 900006 | CT aspects for Enabling Edge Applications | E D G E A P P |  | CP-211196 | Narendranath Durga Tangudu |
| 900034 | CT1 aspects for Enabling Edge Applications | E D G E A P P | C1 | CP-211196 | Narendranath Durga Tangudu |
| 900035 | CT3 aspects for Enabling Edge Applications | E D G E A P P | C3 | CP-211196 | Narendranath Durga Tangudu |
| 830032 | Study on enhancement of support for Edge Computing in 5GC | F S _ e n h _ E C | S2 | SP-200093 | Hui Ni, Huawei |
| 910048 | Enhancement of support for Edge Computing in 5G Core network | e E D G E _ 5 G C |  | SP-201107 | Hui Ni, Huawei Technologies |
| 900016 | Stage 2 of eEDGE_5GC | e E D G E _ 5 G C | S2 | SP-201107 | Hui Ni, Huawei Technologies |
| 910005 | CT aspects of 5G eEDGE | e E D G E _ 5 G C | ct | CP-212021 | Qi Caixia, Huawei |
| 910049 | CT1 aspects of 5G eEDGE | e E D G E _ 5 G C | C1 | CP-212021 | Qi Caixia, Huawei |
| 910050 | CT3 aspects of 5G eEDGE | e E D G E _ 5 G C | C3 | CP-212021 | Qi Caixia, Huawei |
| 910051 | CT4 aspects of 5G eEDGE | e E D G E _ 5 G C | C4 | CP-212021 | Qi Caixia, Huawei |
| 920026 | Security Aspects of Enhancements of Support for Edge Computing in 5GC | e E D G E _ 5 G C | S3 | SP-210423 | Bo Zhang, Huawei Technologies |
| 920019 | Edge Computing Management | E C M | S5 | SP-210388 | Deepanshu Gautam, Samsung |
| 880002 | Study on Security Aspects of Enhancement of Support for Edge Computing in 5GC | F S _ e E D G E _ S e c | S3 | SP-200347 | Bo Zhang, Huawei Technologies |
| 930034 | Charging aspects of Edge Computing | E D G E _ C H | S5 | SP-210861 | Yao, Yizhi, Intel |
| 880030 | Study on charging aspects of Edge Computing | F S _ E D G E _ C H | S5 | SP-200467 | Yizhi Yao, Intel |
| 930011 | Charging aspects of Edge Computing | E D G E _ C H | S5 | SP-210861 | Yao, Yizhi, Intel |
| 870029 | Study on enhancements of edge computing management | F S _ e E D G E _ M g t | S5 | SP-200195 | Joey Chou, Intel |
| 0 | Rel-17 Slicing | - |  | - | - |
| 920058 | Rel-17 Enhancements of 3GPP Northbound Interfaces and Application Layer APIs | N B I 1 7 |  | CP-211197 | Yali Yan, Huawei |
| 920013 | CT1 aspects of NBI17 | N B I 1 7 | C1 | CP-211197 | Yali Yan, Huawei |
| 920053 | CT3 aspects of NBI17 | N B I 1 7 | C3 | CP-211197 | Yali Yan, Huawei |
| 920054 | CT4 aspects of NBI17 | N B I 1 7 | C4 | CP-211197 | Yali Yan, Huawei |
| 920012 | Enhancement of 5G PCC related services in Rel-17 | e n 5 G P c c S e r 1 7 | C3 | CP-211193 | Xiaoyun Zhou, Huawei |
| 900032 | Enhancement of Network Slicing Phase 2 | e N S _ P h 2 |  | SP-200976 | ZTE, Jinguo Zhu |
| 850010 | Study on Enhancement of Network Slicing Phase 2 | F S _ e N S _ P h 2 | S2 | SP-190931 | So, Tricci, ZTE |
| 900011 | Stage 2 for Enhancement of Network Slicing Phase 2 | e N S _ P h 2 | S2 | SP-200976 | ZTE, Jinguo Zhu |
| 920059 | Stage 3 for Enhancement of Network Slicing Phase 2 | e N S _ P h 2 | ct | CP-211091 | Hannah Wang, ZTE |
| 910041 | CT1 aspects of eNS_Ph2 | e N S _ P h 2 | C1 | CP-211091 | Hannah Wang, ZTE |
| 910042 | CT3 aspects of eNS_Ph2 | e N S _ P h 2 | C3 | CP-211091 | Hannah Wang, ZTE |
| 910043 | CT4 aspects of eNS_Ph2 | e N S _ P h 2 | C4 | CP-211091 | Hannah Wang, ZTE |
| 911007 | Enhancement of RAN slicing for NR | N R _ s l i c e | R2 | RP-212534 | CMCC |
| 911107 | Core part: Enhancement of RAN slicing for NR | N R _ s l i c e - C o r e | R2 | RP-212534 | CMCC |
| 860022 | Study on network slice management enhancement | F S _ N S M E N | S5 | SP-200766 | Brendan Hassett, Huawei Technologies Sweden AB, |
| 900023 | Study on Charging Aspects for Network Slicing Phase 2 | F S _ N E T S L I C E _ C H _ P h 2 | S5 | SP-201082 | Matrixx, Gerald Görmer |
| 910023 | Study on enhanced security for network slicing Phase 2 | F S _ e N S 2 _ S E C | S3 | SP-210106 | Zander Lei, Huawei |
| 910026 | Study on network slice management capability exposure | F S _ N S C E | S5 | SP-210131 | Xiaobo Yu, Alibaba Group |
| 910095 | Study on 5G NR UE full stack testing for Network Slicing | F S _ N R _ S l i c e _ T e s t | R5 | RP-211977 | CMCC |
| 0 | Rel-17 Multi-(U)SIM | - |  | - | - |
| 840049 | Support for Multi-USIM Devices | M U S I M |  | SP-190309 | Liao, Ellen C, Intel |
| 830019 | Study on MUSIM | F S _ M U S I M | S1 | SP-190091 | Liao, Ellen C, Intel |
| 840040 | Stage 1 of MUSIM | M U S I M | S1 | SP-190309 | Liao, Ellen C, Intel |
| 820012 | Study on Stage 2 (System Enablers) for MUSIM | F S _ M U S I M | S2 | SP-200297 | Sašo Stojanovski, Intel |
| 900013 | System enablers for Multi-USIM devices | M U S I M | S2 | SP-210091 | Sašo Stojanovski, Intel |
| 910015 | CT aspects of MUSIM | M U S I M | ct | CP-212102 | Vivek Gupta, Intel |
| 910063 | CT1 aspects of Enabling Multi-USIM Devices | M U S I M | C1 | CP-212102 | Vivek Gupta, Intel |
| 910064 | CT4 aspects of Enabling Multi-USIM Devices | M U S I M | C4 | CP-212102 | Vivek Gupta, Intel |
| 900017 | Study on the security of the system enablers for devices having multiple Universal Subscriber Identity Modules (USIM) | F S _ M U S I M _ S E C | S3 | SP-201018 | Abhijeet Kolekar, Intel Corporation |
| 860063 | Support for Multi-SIM devices for LTE/NR | L T E _ N R _ M U S I M |  | RP-212610 | Vivo |
| 860163 | Core part: Support for Multi-SIM devices for LTE/NR | L T E _ N R _ M U S I M - C o r e | R2 | RP-212610 | Vivo |
| 0 | Rel-17 other cross-TSG Features | - |  | - | - |
| 930030 | Support of the 5GMSG Service | F S _ 5 G M A R C H | S6 | SP-200835 | Liu, Yue, China Mobile |
| 840036 | Study on support of the 5GMSG Service | F S _ 5 G M A R C H | S6 | SP-200835 | Liu, Yue, China Mobile |
| 890010 | Study on security aspects of the 5GMSG Service (St1 in Rel16) | F S _ S E C _ 5 G M S G | S3 | SP-200878 | Xiaoting Huang, China Mobile |
| 930005 | Security aspects of the 5GMSG Service | 5 G M S G | S3 | SP-210835 | Xiaoting Huang, China Mobile |
| 930040 | Application Architecture for MSGin5G Service | 5 G M A R C H |  | SP-200832 | Liu, Yue, China Mobile |
| 890026 | Application Architecture for MSGin5G Service | 5 G M A R C H | S6 | SP-200832 | Liu, Yue, China Mobile |
| 930004 | CT aspects for enabling MSGin5G Service | 5 G M A R C H | ct | CP-212106 | Liu, Yue, China Mobile |
| 930041 | CT1 aspects for enabling MSGin5G Service | 5 G M A R C H | C1 | CP-212106 | Liu, Yue, China Mobile |
| 930042 | CT3 aspects for enabling MSGin5G Service | 5 G M A R C H | C3 | CP-212106 | Liu, Yue, China Mobile |
| 930002 | IMS voice service support and network usability guarantee for UE s E-UTRA capability disabled scenario in SA 5GS | I N G _ 5 G S | ct | CP-212231 | Mingxue Li, China Telecom |
| 930047 | CT1 aspects of ING_5GS | I N G _ 5 G S | C1 | CP-212231 | Mingxue Li, China Telecom |
| 930046 | CT6 aspects of ING_5GS | I N G _ 5 G S | C6 | CP-212231 | Mingxue Li, China Telecom |
| 930031 | User Consent for 3GPP services | F S _ U C 3 S | S3 | SP-200885 | Rong Wu, Huawei Technologies |
| 890037 | Study on User Consent for 3GPP services | F S _ U C 3 S | S3 | SP-200885 | Rong Wu, Huawei Technologies |
| 930006 | Security aspects on User Consent for 3GPP services | U C 3 S _ S E C | S3 | SP-210836 | Rong Wu, Huawei Technologies |
| 930032 | Improved support for NSA in the service-based management architecture | N S A _ S B M A | S5 | SP-211121 | Lan Zou, Huawei |
| 910031 | Study on Enhancement of service-based management architecture | F S _ e S B M A | S5 | SP-210136 | Lan Zou, Huawei |
| 930009 | Improved support for NSA in the service-based management architecture | N S A _ S B M A | S5 | SP-210858 | Lan Zou, Huawei |
| 930033 | Access control for management service | M S A C | S5 | SP-210859 | Jing Ping, Nokia |
| 890016 | Study on access control for management service | F S _ M N S A C | S5 | SP-200853 | Jing Ping, Nokia |
| 930010 | Access control for management service | M S A C | S5 | SP-210859 | Jing Ping, Nokia |
| 920060 | NPN support of PWS | N P N _ P W S | S1 | SP-210585 | Francesco Pica, Qualcomm |
| 910007 | Start of Pause of Charging via User Plane | S P O C U P | C4 | CP-210076 | YONG, YANG, Ericsson, frank |
| 910030 | File Management | F I M A | S5 | SP-210135 | Pollakowski, Olaf, Nokia |
| 910024 | Enhancements of 3GPP profiles for cryptographic algorithms and security protocols | e C r y p t P r | S3 | SP-210107 | Pinar Comak, Ericsson |
| 920027 | AKMA TLS protocol profiles | A K M A _ T L S | S3 | SP-210424 | Escott, Adrian, Qualcomm |
| 910025 | User Plane Integrity Protection for LTE | U P I P _ S E C _ L T E | S3 | SP-210105 | Evans, Tim, Vodafone |
| 900002 | Restoration of PDN Connections in PGW-C/SMF Set | R P C P S E T | C4 | CP-203020 | LANDAIS, Bruno, Nokia |
| 900037 | Enhancement of Inter-PLMN Roaming | E o I P R | C4 | CP-203021 | Caixia Qi, Huawei |
| 911037 | Reliable Data Service Serialization Indication | R D S S I |  | SP-190446 | Starsinic, Michael, Convida Wireless LLC |
| 840017 | Stage 2 of RDSSI | R D S S I | S2 | SP-190446 | Starsinic, Michael, Convida Wireless LLC |
| 900005 | CT aspects of RDSSI | R D S S I | C1 | CP-203234 | Vivek Gupta, Intel |
| 900021 | Management Aspects of 5G Network Sharing | M A N S | S5 | SP-201080 | Mingrui Sun, China Unicom |
| 920018 | Study on Management Aspects of 5G Network Sharing | F S _ M A N S | S5 | SP-210387 | Mingrui Sun, China Unicom |
| 900019 | Adapting BEST for use in 5G networks | B E S T _ 5 G | S3 | SP-201020 | Keesmaat, Iko, KPN |
| 920049 | Enhanced Service Enabler Architecture Layer for Verticals | e S E A L |  | SP-200987 | Basavaraj (Basu) Pattan, Samsung |
| 900024 | Stage 2 of eSEAL | e S E A L | S6 | SP-200987 | Basavaraj (Basu) Pattan, Samsung |
| 920006 | CT1 aspects of eSEAL | e S E A L | C1 | CP-212098 | Sapan Shah (Samsung Electronics) |
| 920050 | CT3 aspects of eSEAL | e S E A L | C3 | CP-212098 | Sapan Shah (Samsung Electronics) |
| 900033 | Access Traffic Steering, Switch and Splitting support in the 5G system architecture; Phase 2 | A T S S S _ P h 2 |  | SP-200977 | Apostolis Salkintzis, Lenovo |
| 840084 | Study on ATSSS_Ph2 | F S _ A T S S S _ P h 2 | S2 | SP-200095 | So, Tricci, ZTE |
| 900012 | Stage 2 of ATSSS_Ph2 | A T S S S _ P h 2 | S2 | SP-200977 | Apostolis Salkintzis, Lenovo |
| 910013 | CT aspects of ATSSS_Ph2 | A T S S S _ P h 2 | ct | CP-210136 | ZHOU Xingyue (Joy), ZTE |
| 910056 | CT1 aspects of ATSSS_Ph2 | A T S S S _ P h 2 | C1 | CP-210136 | ZHOU Xingyue (Joy), ZTE |
| 910057 | CT3 aspects of ATSSS_Ph2 | A T S S S _ P h 2 | C3 | CP-210136 | ZHOU Xingyue (Joy), ZTE |
| 910058 | CT4 aspects of ATSSS_Ph2 | A T S S S _ P h 2 | C4 | CP-210136 | ZHOU Xingyue (Joy), ZTE |
| 900031 | Enablers for Network Automation for 5G - phase 2 | e N A _ P h 2 |  | SP-200975 | TBD |
| 840022 | Study on Enablers for Network Automation for 5G - phase 2 | F S _ e N A _ P h 2 | S2 | SP-200098 | Xiaobo Wu, Huawei Technologies |
| 890015 | Study on security aspects of enablers for Network Automation (eNA) for 5GS Phase 2 | F S _ e N A _ S E C | S3 | SP-200722 | Xiaoting Huang, China Mobile |
| 900010 | Stage 2 of eNA_Ph2 | e N A _ P h 2 | S2 | SP-200975 | TBD |
| 910012 | CT aspects of eNA_Ph2 | e N A _ P h 2 | ct | CP-211191 | Huang Zhenning (China Mobile) |
| 910088 | CT3 aspects of eNA_Ph2 | e N A _ P h 2 | C3 | CP-211191 | Huang Zhenning (China Mobile) |
| 910089 | CT4 aspects of eNA_Ph2 | e N A _ P h 2 | C4 | CP-211191 | Huang Zhenning (China Mobile) |
| 930007 | Security aspects of eNA_Ph2 | e N A _ P h 2 | S3 | SP-210837 | Chang Liu, China Mobile |
| 900030 | Proximity based Services in 5GS | 5 G _ P r o S e |  | SP-200972 | TBD |
| 830033 | Study on System enhancement for Proximity based Services in 5GS | F S _ 5 G _ P r o S e | S2 | SP-190443 | Qiang Deng, CATT |
| 900007 | Stage 2 for Proximity based Services in 5GS | 5 G _ P r o S e | S2 | SP-201096 | Qiang Deng, CATT |
| 910018 | CT aspects of proximity based services in 5GS | 5 G _ P r o S e | ct | CP-212105 | Yong Jiang, CATT |
| 910072 | CT1 aspects of 5G_ProSe | 5 G _ P r o S e | C1 | CP-212105 | Yong Jiang, CATT |
| 910073 | CT3 aspects of 5G_ProSe | 5 G _ P r o S e | C3 | CP-212105 | Yong Jiang, CATT |
| 910074 | CT4 aspects of 5G_ProSe | 5 G _ P r o S e | C4 | CP-212105 | Yong Jiang, CATT |
| 920081 | CT6 aspects of 5G_ProSe | 5 G _ P r o S e | C6 | CP-212105 | Yong Jiang, CATT |
| 890018 | Study on charging aspects of Proximity-based Services in 5GS | F S _ 5 G _ P r o s e _ C H | S5 | SP-200767 | Shu, Min, CATT |
| 880005 | Study on Security Aspects of Enhancement for Proximity Based Services in 5GS | F S _ 5 G _ P r o S e _ S e c | S3 | SP-200350 | Wei Zhou, CATT |
| 930008 | Security Aspects of Proximity based Services in 5GS | 5 G _ P r o S e | S3 | SP-211120 | Wei Zhou, CATT |
| 911005 | NR Sidelink Relay | N R _ S L _ r e l a y |  | RP-212601 | OPPO |
| 911105 | Core part: NR Sidelink Relay | N R _ S L _ r e l a y - C o r e | R2 | RP-211050 | OPPO |
| 911205 | Perf. part: NR Sidelink Relay | N R _ S L _ r e l a y - P e r f | R4 | RP-211050 | OPPO |
| 890030 | Authentication and key management for applications based on 3GPP credential in 5G | A K M A |  | SP-190711 | Xiaoting Huang, China Mobile |
| 850021 | SA3 aspects of AKMA | A K M A | S3 | SP-190711 | Xiaoting Huang, China Mobile |
| 890008 | CT aspects of AKMA | A K M A - C T | ct | CP-203107 | Huang Zhenning (China Mobile) |
| 890031 | CT1 aspects of AKMA | A K M A - C T | C1 | CP-203107 | Huang Zhenning (China Mobile) |
| 890032 | CT3 aspects of AKMA | A K M A - C T | C3 | CP-203107 | Huang Zhenning (China Mobile) |
| 890033 | CT4 aspects of AKMA | A K M A - C T | C4 | CP-203107 | Huang Zhenning (China Mobile) |
| 890049 | Enhancement of Private Network support for NG-RAN | N G _ R A N _ P R N _ e n h | R3 | RP-212585 | China Telecom |
| 890149 | Core part: Enhancement of Private Network support for NG-RAN | N G _ R A N _ P R N _ e n h - C o r e | R3 | RP-212585 | China Telecom |
| 840046 | Multimedia Priority Service (MPS) Phase 2 | M P S 2 |  | SP-190305 | Singh, Ray P; Perspecta Labs |
| 780005 | Study on MPS2 | F S _ M P S 2 | S1 | SP-190315 | Singh, Ray P; Vencore Labs |
| 850011 | Study on MPS2, Stage 2 | F S _ M P S 2 _ S t 2 | S2 | SP-190629 | Don Lukacs; Perspecta Labs |
| 840032 | Stage 1 of MPS2 | M P S 2 | S1 | SP-190561 | Singh, Ray P; Perspecta Labs |
| 870002 | Stage 2 of MPS2 | M P S 2 | S2 | SP-200519 | Streijl, Robert, Perspecta Labs |
| 880031 | Stage 3 of MPS2 | M P S 2 | ct | CP-201207 | Peter Monnes, Perspecta Labs |
| 880050 | CT1 aspects of MPS2 | M P S 2 | C1 | CP-201207 | Peter Monnes, Perspecta Labs |
| 880058 | CT3 aspects of MPS2 | M P S 2 | C3 | CP-201207 | Peter Monnes, Perspecta Labs |
| 880051 | CT4 aspects of MPS2 | M P S 2 | C4 | CP-201207 | Peter Monnes, Perspecta Labs |
| 890011 | Mission critical security enhancements phase 2 | M C X S e c 2 | S3 | SP-200879 | Woodward, Tim, Motorola Solutions, Inc, |
| 890070 | PAP/CHAP protocols usage in 5GS | P A P _ C H A P | ct | CP-202251 | Mingxue Li, China Telecom |
| 890071 | CT1 aspects of PAP/CHAP protocols usage in 5GS | P A P _ C H A P | C1 | CP-202251 | Mingxue Li, China Telecom |
| 890072 | CT3 aspects of PAP/CHAP protocols usage in 5GS | P A P _ C H A P | C3 | CP-210188 | Mingxue Li, China Telecom |
| 890001 | Service-based support for SMS in 5GC | S M S _ S B I |  | CP-212023 | HAOUARI, Wafa, Orange |
| 890028 | CT1 aspects of SMS_SBI | S M S _ S B I | C1 | CP-212023 | HAOUARI, Wafa, Orange |
| 890029 | CT4 aspects of SMS_SBI | S M S _ S B I | C4 | CP-212023 | HAOUARI, Wafa, Orange |
| 840048 | Enhanced Relays for Energy eFficiency and Extensive Coverage | R E F E C |  | SP-190307 | Norp, Toon, KPN |
| 810018 | Study on REFEC | F S _ R E F E C | S1 | SP-180785 | Jose Luis Almodovar Chico, KPN |
| 840034 | Stage 1 of REFEC | R E F E C | S1 | SP-190307 | Norp, Toon, KPN |
| 840044 | Network Controlled Interactive Service | N C I S |  | SP-190303 | Ning YANG, oppo |
| 800015 | Study on NCIS | F S _ N C I S | S1 | SP-180341 | YANG Ning, Oppo |
| 840030 | Stage 1 of NCIS | N C I S | S1 | SP-191039 | Ning YANG, oppo |
| 840019 | 5G System Enhancement for Advanced Interactive Services | 5 G _ A I S | S2 | SP-190564 | Lei Yixue, Tencent |
| 890009 | Points for 8K VR 360 Video over 5G | 8 K _ V R _ 5 G | S4 | SP-200667 | Ozgur Oyman, Company: Intel |
| 880043 | Multi-Device and multi-identity Enhancements (MuDE) | M u D E |  | SP-190823 | Adrian.buckley@vivo.com |
| 850041 | Stage 1 of MuDE | M u D E | S1 | SP-190823 | Adrian.buckley@vivo.com |
| 880018 | Stage 3 of MuDE | M u D e | C1 | CP-201162 | Adrian Buckley |
| 920055 | Best Practice of Packet Forwarding Control Protocol (PFCP) | B E P o P | ct | CP-212024 | Song Yue, China Mobile |
| 920056 | CT3 aspects of BEPoP | B E P o P | C3 | CP-212024 | Song Yue, China Mobile |
| 880014 | CT4 aspects of BEPoP | B E P o P | C4 | CP-212024 | Song Yue, China Mobile |
| 880029 | Enhancement of Handover Optimization | E _ H O O | S5 | SP-200466 | Elmdahl, Per, Ericsson |
| 880028 | Management data collection control and discovery | M A D C O L | S5 | SP-200465 | Pollakowski, Olaf, Nokia |
| 880026 | Management of the enhanced tenant concept | e M E M T A N E | S5 | SP-200463 | Zhu, Lei, Huawei |
| 880025 | Enhancements of 5G performance measurements and KPIs | e P M _ K P I _ 5 G | S5 | SP-200462 | Yizhi Yao, Intel |
| 880012 | Extension for headset interface tests of UE | H I n T | S4 | SP-200398 | Jan Reimes, HEAD acoustics GmbH, |
| 840038 | MC services support on IOPS mode of operation | M C I O P S | S6 | SP-190944 | Camilo.solano@ericsson.com |
| 840016 | Supporting Flexible Local Area Data Network | F L A D N | S2 | SP-190563 | LG Electronics, Hyunsook Kim |
| 820003 | Support of Immersive Teleconferencing and Telepresence for Remote Terminals | I T T 4 R T | S4 | SP-180985 | Ozgur Oyman, Intel |
| 830005 | Terminal Audio quality performance and Test methods for Immersive Audio Services | A T I A S | S4 | SP-190040 | Stéphane Ragot, Orange |
| 860012 | Handsets Featuring Non-Traditional Earpieces | H a N T E | S4 | SP-190989 | Schevciw, Andre, Qualcomm |
| 920008 | 5GMS AF Event Exposure | E V E X | S4 | SP-210374 | Lo, Charles, Qualcomm Inc., |
| 920009 | Edge Extensions to the 5G Media Streaming Architecture | 5 G M S _ E D G E | S4 | SP-210375 | Bouazizi, Imed, Qualcomm Inc. |
| 920010 | 5G Multicast-Broadcast User Service Architecture and related 5GMS Extensions | 5 M B U S A | S4 | SP-210376 | TAN, PENG, TELUS |
| 920011 | 8K Television over 5G | 8 K _ T V _ 5 G | S4 | SP-210381 | Thomas Stockhammer |
| 830017 | Verification-modified Calling Name Display | V M O D _ D I S P L A Y | S1 | SP-190085 | Hala Mowafy, Ericsson |
| 850038 | Enhanced Calling Name Service Analytics Interworking | e C N A M _ A n | S1 | SP-190940 | Hala.Mowafy@ericsson.com |
| 880049 | Enhancement for the 5G Control Plane Steering of Roaming for UE in Connected mode | e C P S O R _ C O N |  | CP-210148 | Minokuchi, Atsushi, NTT DOCOMO |
| 850039 | Stage 1 of eCPSOR_CON | e C P S O R _ C O N | S1 | SP-190941 | Minokuchi, Atsushi, NTT DOCOMO |
| 880022 | Stage 3 (CT1) of eCPSOR_CON | e C P S O R _ C O N | C1 | CP-210148 | Al-Bakri Ban, NTT DOCOMO |
| 900036 | Stage 3 (CT4) of eCPSOR_CON | e C P S O R _ C O N | C4 | CP-210148 | Al-Bakri Ban, NTT DOCOMO |
| 920061 | Stage 3 (CT6) of eCPSOR_CON | e C P S O R _ C O N | C6 | CP-210148 | Al-Bakri Ban, NTT DOCOMO |
| 910090 | Integration of Generic Bootstrapping Architecture (GBA) into 5GC | G B A _ 5 G |  | SP-190714 | Vlasios Tsiatsis, Ericsson |
| 850023 | Security aspects of Integration of GBA into 5GC | G B A _ 5 G | S3 | SP-190714 | Vlasios Tsiatsis, Ericsson |
| 910004 | CT aspects of Integration of GBA into SBA | G B A _ 5 G | C4 | CP-210283 | de Gregorio, Jesús (Ericsson) |
| 910047 | Integration of Generic Bootstrapping Architecture (GBA) into 5GC | G B A _ 5 G | S3 | SP-190714 | Vlasios Tsiatsis, Ericsson |
| 770024 | EVS Codec Extension for Immersive Voice and Audio Services | I V A S _ C o d e c | S4 | SP-170611 | Bin Wang, Huawei Technologies Co Ltd |
| 850045 | Support for Minimization of service Interruption | M I N T |  | SP-190814 | SungDuck Chun; LG Electronics |
| 830018 | Study on MINT | F S _ M I N T | S1 | SP-190090 | SungDuck Chun; LG Electronics |
| 850036 | Stage 1 of MINT | M I N T | S1 | SP-190938 | SungDuck Chun; LG Electronics |
| 920062 | Stage 2 of MINT | M I N T | S2 | SP-210582 | Hyunsook Kim, LG Electronics |
| 900004 | Study on the CT aspects of Support for Minimization of service Interruption | F S _ M I N T - C T | C1 | CP-203232 | Sang Min Park, LG Electronics |
| 930003 | Stage 3 of MINT | M I N T | ct | CP-212166 | Sang Min Park |
| 930049 | CT1 aspects of MINT | M I N T | C1 | CP-212166 | Sang Min Park |
| 930045 | CT4 aspects of MINT | M I N T | C4 | CP-212166 | Sang Min Park |
| 930048 | CT6 aspects of MINT | M I N T | C6 | CP-212166 | Sang Min Park |
| 860008 | IMS emergency support for Stand-alone Non-Public Network (SNPN) | I E S N P N | S1 | SP-191038 | Wong, Curt; |
| 870023 | Management of non-public networks (NPN) | O A M _ N P N | S5 | SP-200189 | ZHANG, Kai, Huawei |
| 860016 | Assurance Specification for IMS | S C A S _ I M S | S3 | SP-191128 | Bo Zhang, Huawei Technologies |
| 870020 | Security Assurance Specification for 5G (eSCAS_5G) | e S C A S _ 5 G | S3 | SP-200149 | Rong Wu, Huawei Technologies Co. |
| 890013 | eSCAS_5G for Network Slice-Specific Authentication and Authorization Function (NSSAAF) | S C A S _ 5 G _ N S S A A F | S3 | SP-200720 | Rong Wu, Huawei Technologies Co., Ltd. |
| 870017 | eSCAS_5G for Non-3GPP InterWorking Function | S C A S _ 5 G _ N 3 I W F | S3 | SP-200146 | Feng Gao, China Unicom |
| 870018 | eSCAS_5G for 5G NWDAF | S C A S _ 5 G _ N W D A F | S3 | SP-200147 | QI Minpeng, China Mobile |
| 870019 | eSCAS_5G for Service Communication Proxy | S C A S _ 5 G _ S E C O P | S3 | SP-200148 | Wei Lu, Nokia |
| 880003 | eSCAS_5Gfor Inter PLMN UP Security | S C A S _ 5 G _ I P U P S | S3 | SP-200348 | Jin PENG, ZTE Corporation |
| 870022 | Enhancements on EE for 5G networks | E E 5 G P L U S | S5 | SP-200188 | Jean-Michel CORNILY, ORANGE |
| 870024 | Enhancement on Management Aspects of 5G Service-Level Agreement | E M A 5 S L A | S5 | SP-210860 | SHI, Xiaonan, CMCC |
| 870025 | Management of MDT enhancement in 5G | e _ 5 G M D T | S5 | SP-200191 | Ayani, Zhulia, Ericsson |
| 870026 | Additional NRM features | a d N R M | S5 | SP-200192 | Jing Ping, Nokia |
| 870027 | Enhancement of QoE Measurement Collection | e Q o E | S5 | SP-200193 | Petersen, Robert, Ericsson |
| 860061 | Study on NR QoE (Quality of Experience) management and optimizations for diverse services | F S _ N R _ Q o E | R3 | RP-193256 | China Unicom |
| 870030 | Enhanced Closed loop SLS Assurance | e C O S L A | S5 | SP-200196 | Groenendijk, Jan, Ericsson |
| 870034 | Enhanced Mission Critical Push-to-talk architecture phase 3 | e n h 3 M C P T T |  | SP-200108 | Dom Lazara; Motorola Solutions |
| 870016 | Stage 2 aspects of enh3MCPTT | e n h 3 M C P T T | S6 | SP-200108 | Dom Lazara; Motorola Solutions |
| 900039 | Stage 3 aspects of enh3MCPTT | e n h 3 M C P T T - C T | C1 | CP-202188 |  |
| 840028 | IMS Charging in 5G System Architecture | 5 G S I M S C H | S5 | SP-190367 | Jahangir, Zeeshan, T-Mobile USA |
| 810027 | Intent driven management service for mobile network | I D M S _ M N | S5 | SP-180899 | Zou Lan, Huawei |
| 820006 | Study on User Plane Integrity Protection | F S _ U P _ I P _ S e c | S3 | SP-181035 | Evans, Tim, Vodafone |
| 890012 | Enhancements to User Plane Integrity Protection Support in 5GS | e U P I P _ S E C | S3 | SP-200719 | Anand Palanigounder, Qualcomm |
| 820035 | Discovery of management services in 5G | 5 G D M S | S5 | SP-200770 | Brendan Hassett, Huawei Technologies Sweden AB |
| 860024 | Network policy managementfor 5G mobile networks | N P M | S5 | SP-191211 | China Mobile Shasha Guo |
| 860025 | Lawful Interception Rel-17 | L I 1 7 | S3 | SP-190983 | Alex Leadbeater, BT |
| 850047 | (Small) Technical Enhancements and Improvements for Rel-17 | T E I 1 7 |  |  |  |
| 870003 | IMS Optimization for HSS Group ID in an SBA environment | T E I 1 7 _ I M S G I D | S2 | SP-200452 | Foti, George, Ericsson |
| 910044 | Same PCF Selection For AMF and SMF | T E I 1 7 _ S P S F A S |  | SP-200447 | Heng, Nie, China Telecom |
| 880016 | Stage 2 for TEI17_SPSFAS | T E I 1 7 _ S P S F A S | S2 | SP-200447 | Heng, Nie, China Telecom |
| 910003 | CT aspects on TEI17_SPSFAS | T E I 1 7 _ S P S F A S | ct | CP-211184 | Yue Sun, China Telecom |
| 910045 | CT3 aspects on TEI17_SPSFAS | T E I 1 7 _ S P S F A S | C3 | CP-211184 | Yue Sun, China Telecom |
| 910046 | CT4 aspects on TEI17_SPSFAS | T E I 1 7 _ S P S F A S | C4 | CP-211184 | Yue Sun, China Telecom |
| 930036 | System enhancement for Redundant PDU Session | T E I 1 7 _ S E _ R P S |  | SP-200448 | Devaki Chandramouli |
| 880017 | Stage 2 for TEI17_SE_RPS | T E I 1 7 _ S E _ R P S | S2 | SP-200448 | Devaki Chandramouli |
| 930001 | CT aspects of TEI17_SE_RPS | T E I 1 7 _ S E _ R P S | ct | CP-212099 | WON, Sung Hwan, Nokia |
| 930037 | CT1 aspects of TEI17_SE_RPS | T E I 1 7 _ S E _ R P S | C1 | CP-212099 | WON, Sung Hwan, Nokia |
| 930038 | CT3 aspects of TEI17_SE_RPS | T E I 1 7 _ S E _ R P S | C3 | CP-212099 | WON, Sung Hwan, Nokia |
| 930039 | CT4 aspects of TEI17_SE_RPS | T E I 1 7 _ S E _ R P S | C4 | CP-212099 | WON, Sung Hwan, Nokia |
| 910078 | Support for Signed Attestation for Priority and Emergency Sessions | T E I 1 7 _ S A P E S |  | SP-200453 | Foti, George, Ericsson |
| 870004 | Stage 2 for TEI17_SAPES | T E I 1 7 _ S A P E S | S2 | SP-200453 | Foti, George, Ericsson |
| 910020 | CT aspects on TEI17_SAPES | T E I 1 7 _ S A P E S | ct | CP-210272 | Axell, Jörgen, Ericsson |
| 910079 | CT1 aspects on TEI17_SAPES | T E I 1 7 _ S A P E S | C1 | CP-210272 | Axell, Jörgen, Ericsson |
| 910080 | CT3 aspects on TEI17_SAPES | T E I 1 7 _ S A P E S | C3 | CP-210272 | Axell, Jörgen, Ericsson |
| 910081 | Dynamically Changing AM Policies in the 5GC | T E I 1 7 _ D C A M P |  | SP-200446 | Pancorbo, Belen (Ericsson) |
| 870005 | Stage 2 for TEI17_DCAMP | T E I 1 7 _ D C A M P | S2 | SP-200446 | Pancorbo, Belen (Ericsson) |
| 910009 | CT aspects on TEI17_DCAMP | T E I 1 7 _ D C A M P | ct | CP-212163 | Garcia Azorero, Fuencisla. Ericsson |
| 910082 | CT3 aspects on TEI17_DCAMP | T E I 1 7 _ D C A M P | C3 | CP-212163 | Garcia Azorero, Fuencisla. Ericsson |
| 910083 | CT4 aspects on TEI17_DCAMP | T E I 1 7 _ D C A M P | C4 | CP-212163 | Garcia Azorero, Fuencisla. Ericsson |
| 870006 | IP address pool information from UDM | T E I 1 7 _ I P U | S2 | SP-200454 | Stefan Rommer |
| 910084 | Dynamic management of group-based event monitoring | T E I 1 7 _ G E M |  | SP-200455 | Qian XB Chen, Ericsson |
| 870007 | Stage 2 for TEI17_GEM | T E I 1 7 _ G E M | S2 | SP-200455 | Qian XB Chen, Ericsson |
| 910010 | CT aspects on TEI17_GEM | T E I 1 7 _ G E M | ct | CP-212165 | Maria Tianmei, Liang, Ericsson |
| 910085 | CT3 aspects on TEI17_GEM | T E I 1 7 _ G E M | C3 | CP-212165 | Maria Tianmei, Liang, Ericsson |
| 910086 | CT4 aspects on TEI17_GEM | T E I 1 7 _ G E M | C4 | CP-212165 | Maria Tianmei, Liang, Ericsson |
| 920041 | Support of different slices over different Non 3GPP access | T E I 1 7 _ N 3 S L I C E |  | SP-200456 | Laurent Thiebaut, Nokia |
| 870008 | Stage 2 for TEI17_N3SLICE | T E I 1 7 _ N 3 S L I C E | S2 | SP-200456 | Laurent Thiebaut, Nokia |
| 930099 | CT1 aspects of TEI17_N3SLICE | T E I 1 7 _ N 3 S L I C E | C1 | CP-211089 | LANDAIS, Bruno, Nokia |
| 920001 | CT4 aspects of TEI17_N3SLICE | T E I 1 7 _ N 3 S L I C E | C4 | CP-211089 | LANDAIS, Bruno, Nokia |
| 910087 | N7/N40 Interfaces Enhancements to Support GERAN and UTRAN | T E I 1 7 _ N I E S G U |  | SP-200457 | Aihua, Li, China Mobile |
| 870009 | Stage 2 for TEI17_NIESGU | T E I 1 7 _ N I E S G U | S2 | SP-200457 | Aihua, Li, China Mobile |
| 890019 | SA5 aspects of N40 Interface Enhancements to Support GERAN and UTRAN | T E I 1 7 _ N I E S G U | S5 | SP-200854 | Dong, Jia, China Mobile |
| 910011 | CT3 aspects of N7 Interfaces Enhancements to Support GERAN and UTRAN | T E I 1 7 _ N I E S G U | C3 | CP-211194 | Huang Zhenning (China Mobile) |
| 920082 | CT4 aspects of N7 Interfaces Enhancements to Support GERAN and UTRAN | T E I 1 7 _ N I E S G U | C4 | CP-211194 | Huang Zhenning (China Mobile) |
| 880019 | Stage-3 5GS NAS protocol development 17 | 5 G P r o t o c 1 7 | C1 | CP-201163 | Sedlacek, Ivo, Ericsson |
| 880044 | Stage-3 5GS NAS protocol development 17 general aspects | 5 G P r o t o c 1 7 | C1 | CP-201163 | Sedlacek, Ivo, Ericsson |
| 880045 | Stage-3 5GS NAS protocol development 17 non-IETF aspects | 5 G P r o t o c 1 7 - n o n 3 G P P | C1 | CP-201163 | Sedlacek, Ivo, Ericsson |
| 880021 | Stage-3 SAE Protocol Development | S A E S 1 7 | C1 | CP-201165 | Aghili, Behrouz, InterDigital Communications |
| 880046 | Stage-3 SAE Protocol Development general aspects | S A E S 1 7 | C1 | CP-201165 | Aghili, Behrouz, InterDigital Communications |
| 880047 | Stage-3 SAE Protocol Development CSFB aspects | S A E S 1 7 - C S F B | C1 | CP-201165 | Aghili, Behrouz, InterDigital Communications |
| 880048 | Stage-3 SAE Protocol Development non 3GPP aspects | S A E S 1 7 - n o n 3 G P P | C1 | CP-201165 | Aghili, Behrouz, InterDigital Communications |
| 911034 | IMS Stage-3 IETF Protocol Alignment | I M S P r o t o c 1 7 | ct | CP-201167 | Gkatzikis, Lazaros(Nokia) |
| 880023 | CT1 aspects of IMS Stage-3 IETF Protocol Alignment | I M S P r o t o c 1 7 | C1 | CP-201167 | Gkatzikis, Lazaros(Nokia) |
| 911035 | CT3 aspects of IMS Stage-3 IETF Protocol Alignment | I M S P r o t o c 1 7 | C3 | CP-201167 | Gkatzikis, Lazaros(Nokia) |
| 880032 | Packet Flow Description (PFD) management enhancement | p f d M a n E n h | C3 | CP-210183 | Xiaoyun Zhou, Huawei |
| 880013 | Service Based Interface Protocol Improvements Release 17 | S B I P r o t o c 1 7 |  | CP-211088 | Song Yue, China Mobile |
| 880053 | CT3 aspects of SBIProtoc17 | S B I P r o t o c 1 7 | C3 | CP-211088 | Song Yue, China Mobile |
| 880054 | CT4 aspects of SBIProtoc17 | S B I P r o t o c 1 7 | C4 | CP-211088 | Song Yue, China Mobile |
| 850028 | Study on enhancement of Management Data Analytics Service | F S _ e M D A S | S5 | SP-190930 | Yao, Yizhi, Intel |
| 910027 | Enhancements of Management Data Analytics Service | e M D A S | S5 | SP-210132 | Yao, Yizhi, Intel |
| 0 | Rel-17 Other NR-only improvements | - |  | - | - |
| 911008 | NR QoE magement and optimizations for diverse services | N R _ Q o E |  | RP-211406 | China Unicom |
| 911108 | Core part: NR QoE management and optimizations for diverse services | N R _ Q o E - C o r e | R3 | RP-211406 | China Unicom |
| 911009 | Rel-17 Power Class 2 UE for NR inter-band CA with or without SUL configurations with x (16>=x>2) bands DL and y (y=1, 2) bands UL | N R _ U E _ P C 2 _ R 1 7 _ C A D C _ S U L _ x B D L _ y B U L |  | RP-212182 | Huawei |
| 911109 | Core part: NR_UE_PC2_R17_CADC_SUL_xBDL_yBUL | N R _ U E _ P C 2 _ R 1 7 _ C A D C _ S U L _ x B D L _ y B U L - C o r e | R4 | RP-212182 | Huawei |
| 911209 | Perf. part: NR_UE_PC2_R17_CADC_SUL_xBDL_yBUL | N R _ U E _ P C 2 _ R 1 7 _ C A D C _ S U L _ x B D L _ y B U L - P e r f | R4 | RP-212182 | Huawei |
| 911010 | Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC) | N R _ F R 1 _ T R P _ T R S |  | RP-212030 | Vivo |
| 911110 | Core part: NR_FR1_TRP_TRS | N R _ F R 1 _ T R P _ T R S - C o r e | R4 | RP-212030 | Vivo |
| 911210 | Perf. part: NR_FR1_TRP_TRS | N R _ F R 1 _ T R P _ T R S - P e r f | R4 | RP-212030 | Vivo |
| 911012 | High power UE (power class 1.5) for NR band n79 | N R _ U E _ P C 1 _ 5 _ n 7 9 |  | RP-210843 | CMCC |
| 911112 | Core part: NR_UE_PC1_5_n79 | N R _ U E _ P C 1 _ 5 _ n 7 9 - C o r e | R4 | RP-210843 | CMCC |
| 911212 | Perf. part: NR_UE_PC1_5_n79 | N R _ U E _ P C 1 _ 5 _ n 7 9 - P e r f | R4 | RP-210843 | CMCC |
| 930052 | UE Conformance - High power UE (power class 1.5) for NR band n79 | N R _ U E _ P C 1 _ 5 _ n 7 9 - U E C o n T e s t | R5 | RP-211979 | CMCC |
| 911013 | High power UE (power class 2) for NR band n34 | N R _ U E _ P C 2 _ n 3 4 |  | RP-210844 | CMCC |
| 911113 | Core part: NR_UE_PC2_n34 | N R _ U E _ P C 2 _ n 3 4 - C o r e | R4 | RP-210844 | CMCC |
| 911213 | Perf. part: NR_UE_PC2_n34 | N R _ U E _ P C 2 _ n 3 4 - P e r f | R4 | RP-210844 | CMCC |
| 930053 | UE Conformance - High power UE (power class 2) for NR band n34 | N R _ U E _ P C 2 _ n 3 4 - U E C o n T e s t | R5 | RP-211980 | CMCC |
| 911014 | High power UE (power class 2) for NR band n39 | N R _ U E _ P C 2 _ n 3 9 |  | RP-210845 | CMCC |
| 911114 | Core part: NR_UE_PC2_n39 | N R _ U E _ P C 2 _ n 3 9 - C o r e | R4 | RP-210845 | CMCC |
| 911214 | Perf. part: NR_UE_PC2_n39 | N R _ U E _ P C 2 _ n 3 9 - P e r f | R4 | RP-210845 | CMCC |
| 930054 | UE Conformance - High power UE (power class 2) for NR band n39 | N R _ U E _ P C 2 _ n 3 9 - U E C o n T e s t | R5 | RP-211981 | CMCC |
| 911015 | Introduction of FR2 FWA (Fixed Wireless Access) UE with maximum TRP of 23dBm for band n259 | N R _ F R 2 _ F W A _ B n 2 5 9 |  | RP-210875 | SoftBank Corp., KDDI, NTT DOCOMO, Rakuten Mobile, Qualcomm |
| 911115 | Core part: NR_FR2_FWA_Bn259 | N R _ F R 2 _ F W A _ B n 2 5 9 - C o r e | R4 | RP-210875 | SoftBank Corp., KDDI, NTT DOCOMO, Rakuten Mobile, Qualcomm |
| 911215 | Perf. part: NR_FR2_FWA_Bn259 | N R _ F R 2 _ F W A _ B n 2 5 9 - P e r f | R4 | RP-210875 | SoftBank Corp., KDDI, NTT DOCOMO, Rakuten Mobile, Qualcomm |
| 911006 | NR Uplink Data Compression (UDC) | N R _ U D C |  | RP-211203 | CATT |
| 911106 | Core part: NR_UDC | N R _ U D C - C o r e | R2 | RP-211203 | CATT |
| 910097 | Study on optimizations of pi/2 BPSK uplink power in NR | F S _ N R _ O p t _ p i 2 B P S K | R4 | RP-210910 | Huawei |
| 910096 | Study on band combination handling in RAN4 | F S _ N R _ E N D C _ c o m b o _ r u l e s | R4 | RP-211146 | ZTE |
| 880078 | Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) requirements for NR UEs | N R _ M I M O _ O T A |  | RP-201800 | CAICT |
| 880178 | Core part: NR_MIMO_OTA | N R _ M I M O _ O T A - C o r e | R4 | RP-201800 | CAICT |
| 880278 | Perf. Part: NR_MIMO_OTA | N R _ M I M O _ O T A - P e r f | R4 | RP-201800 | CAICT |
| 870064 | Introduction of FR2 FWA (Fixed Wireless Access) UE with maximum TRP (Total Radiated Power) of 23dBm for band n257 and n258 | N R _ F R 2 _ F W A _ B n 2 5 7 _ B n 2 5 8 |  | RP-202565 | Softbank |
| 870164 | Core part: NR_FR2_FWA_Bn257_Bn258 | N R _ F R 2 _ F W A _ B n 2 5 7 _ B n 2 5 8 - C o r e | R4 | RP-202565 | Softbank |
| 870264 | Perf. Part: NR_FR2_FWA_Bn257_Bn258 | N R _ F R 2 _ F W A _ B n 2 5 7 _ B n 2 5 8 - P e r f | R4 | RP-202565 | Softbank |
| 880092 | Adding channel bandwidth support to existing NR bands | N R _ b a n d s _ R 1 7 _ B W s |  | RP-212531 | Ericsson |
| 880192 | Core part: NR_bands_R17_BWs | N R _ b a n d s _ R 1 7 _ B W s - C o r e | R4 | RP-212531 | Ericsson |
| 880093 | Introduction of channel bandwidths 35MHz and 45MHz for NR FR1 | N R _ F R 1 _ 3 5 M H z _ 4 5 M H z _ B W |  | RP-211386 | Huawei |
| 880193 | Core part: NR_FR1_35MHz_45MHz_BW | N R _ F R 1 _ 3 5 M H z _ 4 5 M H z _ B W - C o r e | R4 | RP-211386 | Huawei |
| 880293 | Perf. Part: NR_FR1_35MHz_45MHz_BW | N R _ F R 1 _ 3 5 M H z _ 4 5 M H z _ B W - P e r f | R4 | RP-211386 | Huawei |
| 880097 | SAR schemes for UE power class 2 (PC2) for NR inter-band Carrier Aggregation and supplemental uplink (SUL) configurations with 2 bands UL | N R _ S A R _ P C 2 _ i n t e r B _ S U L _ 2 B U L |  | RP-212530 | China Telecom |
| 880197 | Core part: NR_SAR_PC2_interB_SUL_2BUL | N R _ S A R _ P C 2 _ i n t e r B _ S U L _ 2 B U L - C o r e | R4 | RP-212530 | China Telecom |
| 880297 | Perf. Part: NR_SAR_PC2_interB_SUL_2BUL | N R _ S A R _ P C 2 _ i n t e r B _ S U L _ 2 B U L - P e r f | R4 | RP-212530 | China Telecom |
| 920065 | UE Conformance - SAR schemes for UE power class 2 (PC2) for NR inter-band Carrier Aggregation and supplemental uplink (SUL) configurations with 2 bands UL | N R _ S A R _ P C 2 _ i n t e r B _ S U L _ 2 B U L - U E C o n T e s t | R5 | RP-211139 | China Telecom |
| 900061 | NR coverage enhancements | N R _ c o v _ e n h |  | RP-211566 | China Telecom |
| 860036 | Study on on NR coverage enhancements | F S _ N R _ c o v _ e n h | R1 | RP-200861 | China Telecom |
| 900161 | Core part: NR_cov_enh | N R _ c o v _ e n h - C o r e | R1 | RP-211566 | China Telecom |
| 900261 | Perf. part: NR_cov_enh | N R _ c o v _ e n h - P e r f | R4 | RP-211566 | China Telecom |
| 900062 | Support of reduced capability NR devices | N R _ r e d c a p |  | RP-211574 | Ericsson |
| 860035 | Study on support of reduced capability NR devices | F S _ N R _ r e d c a p | R1 | RP-202704 | Ericsson |
| 900162 | Core part: NR_redcap | N R _ r e d c a p - C o r e | R1 | RP-211574 | Ericsson |
| 900262 | Perf. part: NR_redcap | N R _ r e d c a p - P e r f | R4 | RP-211574 | Ericsson |
| 930018 | Architecture Enhancement for NR Reduced Capability Devices | A R C H _ N R _ R E D C A P | S2 | SP-211100 | Aihua Li, China Mobile |
| 900064 | Additional NR bands for UL-MIMO power class 3 (PC3) in Rel-17 | N R _ b a n d s _ U L _ M I M O _ P C 3 _ R 1 7 |  | RP-212184 | Huawei |
| 900164 | Core part: NR_bands_UL_MIMO_PC3_R17 | N R _ b a n d s _ U L _ M I M O _ P C 3 _ R 1 7 - C o r e | R4 | RP-212184 | Huawei |
| 900264 | Perf. part: NR_bands_UL_MIMO_PC3_R17 | N R _ b a n d s _ U L _ M I M O _ P C 3 _ R 1 7 - P e r f | R4 | RP-212184 | Huawei |
| 900167 | Introduction of bandwidth combination set 4 (BCS4) for NR | N R _ B C S 4 - C o r e | R4 | RP-202832 | Ericsson |
| 900068 | Downlink interruption for NR and EN-DC band combinations to conduct dynamic Tx Switching in Uplink | D L _ i n t r p t _ c o m b o s _ T x S W _ R 1 7 |  | RP-210478 | China Telecom |
| 900168 | Core part: DL_intrpt_combos_TxSW_R17 | D L _ i n t r p t _ c o m b o s _ T x S W _ R 1 7 - C o r e | R4 | RP-202885RP-210478 | China Telecom |
| 900268 | Perf. part: DL_intrpt_combos_TxSW_R17 | D L _ i n t r p t _ c o m b o s _ T x S W _ R 1 7 - P e r f | R4 | RP-202885RP-210478 | China Telecom |
| 900069 | High-power UE (power class 1.5) operation in NR bands n77 and n78 | H P U E _ P C 1 _ 5 _ n 7 7 _ n 7 8 |  | RP-202912 | Qualcomm |
| 900169 | Core part: HPUE_PC1_5_n77_n78 | H P U E _ P C 1 _ 5 _ n 7 7 _ n 7 8 - C o r e | R4 | RP-202912 | Qualcomm |
| 930055 | UE Conformance - High-power UE (power class 1.5) operation in NR bands n77 and n78 | H P U E _ P C 1 _ 5 _ n 7 7 _ n 7 8 - U E C o n T e s t | R5 | RP-212581 | Verizon |
| 900070 | NR repeaters | N R _ r e p e a t e r s |  | RP-212129 | Qualcomm |
| 900170 | Core part: HPUE_PC1_5_n77_n78 | N R _ r e p e a t e r s - C o r e | R4 | RP-212129 | Qualcomm |
| 930250 | Perf. part: NR repeaters | N R _ r e p e a t e r s - P e r f | R4 | RP-212129 | Qualcomm |
| 930056 | Increasing UE power high limit for CA and DC | P o w e r _ L i m i t _ C A _ D C | R4 | RP-212622 | Qualcomm |
| 930156 | Core part: Increasing UE power high limit for CA and DC | P o w e r _ L i m i t _ C A _ D C - C o r e | R4 | RP-212622 | Qualcomm |
| 930057 | Introduction of upper 700MHz A block E-UTRA band for the US | L T E _ u p p e r _ 7 0 0 M H z _ A | R4 | RP-212618 | Puloli |
| 930157 | Core part: Introduction of upper 700MHz A block E-UTRA band for the US | L T E _ u p p e r _ 7 0 0 M H z _ A - C o r e | R4 | RP-212618 | Puloli |
| 930257 | Perf. part: Introduction of upper 700MHz A block E-UTRA band for the US | L T E _ u p p e r _ 7 0 0 M H z _ A - P e r f | R4 | RP-212618 | Puloli |
| 930058 | High power UE (power class 2) for one NR FDD band | N R _ P C 2 _ U E _ F D D | R4 | RP-212633 | China Unicom |
| 930158 | Core part: High power UE (power class 2) for one NR FDD band | N R _ P C 2 _ U E _ F D D - C o r e | R4 | RP-212633 | China Unicom |
| 930258 | Perf. part: High power UE (power class 2) for one NR FDD band | N R _ P C 2 _ U E _ F D D - P e r f | R4 | RP-212633 | China Unicom |
| 0 | Rel-17 NR Carrier Aggregation | - |  | - | - |
| 0 | Rel-17 NR intra band Carrier Aggregation (itrabCA) | - |  | - | - |
| 881005 | NR itrabCA for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y) | N R _ C A _ R 1 7 _ I n t r a |  | RP-211757 | Ericsson |
| 881105 | Core part: NR_CA_R17_Intra | N R _ C A _ R 1 7 _ I n t r a - C o r e | R4 | RP-211757 | Ericsson |
| 881205 | Perf. Part: NR_CA_R17_Intra | N R _ C A _ R 1 7 _ I n t r a - P e r f | R4 | RP-211757 | Ericsson |
| 0 | Rel-17 NR inter band Carrier Aggregation (iterbCA) | - |  | - | - |
| 900063 | High power UE for NR TDD intra-band Carrier Aggregation in frequency range FR1 | N R _ i n t r a _ H P U E _ R 1 7 |  | RP-212180 | Huawei |
| 900163 | Core part: NR_intra_HPUE_R17 | N R _ i n t r a _ H P U E _ R 1 7 - C o r e | R4 | RP-212180 | Huawei |
| 900263 | Perf. Part: NR_intra_HPUE_R17 | N R _ i n t r a _ H P U E _ R 1 7 - P e r f | R4 | RP-212180 | Huawei |
| 881006 | NR iterbCA/Dual Connectivity for 2 bands DL with x bands UL (x=1,2) | N R _ C A D C _ R 1 7 _ 2 B D L _ x B U L |  | RP-212511 | ZTE |
| 881106 | Core part: NR_CADC_R17_2BDL_xBUL | N R _ C A D C _ R 1 7 _ 2 B D L _ x B U L - C o r e | R4 | RP-212511 | ZTE |
| 881206 | Perf. part: NR_CADC_R17_2BDL_xBUL | N R _ C A D C _ R 1 7 _ 2 B D L _ x B U L - P e r f | R4 | RP-212511 | ZTE |
| 881007 | NR iterbCA for 3 bands DL with 1 band UL | N R _ C A _ R 1 7 _ 3 B D L _ 1 B U L |  | RP-212239 | CATT |
| 881107 | Core part: NR_CA_R17_3BDL_1BUL | N R _ C A _ R 1 7 _ 3 B D L _ 1 B U L - C o r e | R4 | RP-212239 | CATT |
| 881207 | Perf. part: NR_CA_R17_3BDL_1BUL | N R _ C A _ R 1 7 _ 3 B D L _ 1 B U L - P e r f | R4 | RP-212239 | CATT |
| 881008 | NR iterbCA/Dual Connectivity for 3 bands DL with 2 bands UL | N R _ C A D C _ R 1 7 _ 3 B D L _ 2 B U L |  | RP-212512 | ZTE |
| 881108 | Core part: NR_CADC_R17_3BDL_2BUL | N R _ C A D C _ R 1 7 _ 3 B D L _ 2 B U L - C o r e | R4 | RP-212512 | ZTE |
| 881208 | Perf. part: NR_CADC_R17_3BDL_2BUL | N R _ C A D C _ R 1 7 _ 3 B D L _ 2 B U L - P e r f | R4 | RP-212512 | ZTE |
| 881009 | NR iterbCA for 4 bands DL with 1 band UL | N R _ C A _ R 1 7 _ 4 B D L _ 1 B U L |  | RP-211759 | Ericsson |
| 881109 | Core part: NR_CA_R17_4BDL_1BUL | N R _ C A _ R 1 7 _ 4 B D L _ 1 B U L - C o r e | R4 | RP-211759 | Ericsson |
| 881209 | Perf. part: NR_CA_R17_4BDL_1BUL | N R _ C A _ R 1 7 _ 4 B D L _ 1 B U L - P e r f | R4 | RP-211759 | Ericsson |
| 881010 | NR iterbCA/Dual connectivity for DL 4 bands and 2UL bands | N R _ C A D C _ R 1 7 _ 4 B D L _ 2 B U L |  | RP-212110 | Samsung |
| 881110 | Core part: NR_CADC_R17_4BDL_2BUL | N R _ C A D C _ R 1 7 _ 4 B D L _ 2 B U L - C o r e | R4 | RP-212110 | Samsung |
| 881210 | Perf. part: NR_CADC_R17_4BDL_2BUL | N R _ C A D C _ R 1 7 _ 4 B D L _ 2 B U L - P e r f | R4 | RP-212110 | Samsung |
| 881011 | NR iterbCA for 5 bands DL with x bands UL (x=1, 2) | N R _ C A D C _ R 1 7 _ 5 B D L _ x B U L |  | RP-212176 | Huawei |
| 881111 | Core part: NR_CADC_R17_5BDL_xBUL | N R _ C A D C _ R 1 7 _ 5 B D L _ x B U L - C o r e | R4 | RP-212176 | Huawei |
| 881211 | Perf. part: NR_CADC_R17_5BDL_xBUL | N R _ C A D C _ R 1 7 _ 5 B D L _ x B U L - P e r f | R4 | RP-212176 | Huawei |
| 890054 | Rel-17 High power UE for NR inter-band Carrier Aggregation with 2 bands downlink and x bands uplink (x=1,2) | N R _ P C 2 _ C A _ R 1 7 _ 2 B D L _ 2 B U L |  | RP-211831 | China Telecom |
| 890154 | Core part: NR_PC2_CA_R17_2BDL_2BUL | N R _ P C 2 _ C A _ R 1 7 _ 2 B D L _ 2 B U L - C o r e | R4 | RP-211831 | China Telecom |
| 890254 | Perf. part: NR_PC2_CA_R17_2BDL_2BUL | N R _ P C 2 _ C A _ R 1 7 _ 2 B D L _ 2 B U L - P e r f | R4 | RP-211831 | China Telecom |
| 920066 | UE Conformance - Rel-17 High power UE for NR inter-band Carrier Aggregation with 2 bands downlink and x bands uplink (x=1,2) | N R _ P C 2 _ C A _ R 1 7 _ 2 B D L _ 2 B U L - U E C o n T e s t | R5 | RP-211140 | China Telecom |
| 0 | Rel-17 NR and LTE | - |  | - | - |
| 920072 | LTE/NR spectrum sharing in Band 34/n34 and Band 39/n39 | D S S _ L T E _ B 3 4 _ N R _ B n 3 4 _ L T E _ B 3 9 _ N R _ B n 3 9 | R4 |  | CMCC |
| 920172 | Core part: LTE/NR spectrum sharing in Band 34/n34 and Band 39/n39 | D S S _ L T E _ B 3 4 _ N R _ B n 3 4 _ L T E _ B 3 9 _ N R _ B n 3 9 - C o r e | R4 | RP-211996 | CMCC |
| 920073 | High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands | L T E _ N R _ H P U E _ F W V M | R4 | RP-212533 | Nokia |
| 920173 | Core part: High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands | L T E _ N R _ H P U E _ F W V M - C o r e | R4 | RP-212533 | Nokia |
| 920273 | Perf. part: High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands | L T E _ N R _ H P U E _ F W V M - P e r f | R4 | RP-212533 | Nokia |
| 920074 | Rel-17 Dual Connectivity (DC) of x bands (x=1,2) LTE inter-band CA (xDL1UL) and 4 bands NR inter-band CA (4DL1UL) | D C _ R 1 7 _ x B L T E _ 4 B N R _ y D L 2 U L | R4 | RP-212606 | Huawei |
| 920174 | Core part: Rel-17 Dual Connectivity (DC) of x bands (x=1,2) LTE inter-band CA (xDL1UL) and 4 bands NR inter-band CA (4DL1UL) | D C _ R 1 7 _ x B L T E _ 4 B N R _ y D L 2 U L - C o r e | R4 | RP-212606 | Huawei |
| 920274 | Perf. part: Rel-17 Dual Connectivity (DC) of x bands (x=1,2) LTE inter-band CA (xDL1UL) and 4 bands NR inter-band CA (4DL1UL) | D C _ R 1 7 _ x B L T E _ 4 B N R _ y D L 2 U L - P e r f | R4 | RP-212606 | Huawei |
| 911019 | Addition of MSD (Maximum Sensitivity Degradation) for inter-band EN-DC combinations (1 band LTE+1 band NR FR1) due to added channel bandwidths | E N D C _ R 1 7 _ 1 B L T E _ 1 B N R _ M S |  | RP-212187 | Huawei, HiSilicon |
| 911119 | Core part: ENDC_R17_1BLTE_1BNR_MS | E N D C _ R 1 7 _ 1 B L T E _ 1 B N R _ M S D - C o r e |  | RP-212187 | Huawei, HiSilicon |
| 911011 | Rel-17 Power Class 2 for EN-DC with x LTE bands + y NR band(s) in DL and with 1 LTE band +1 TDD NR band in UL (either x= 2, 3, y=1 or x=1, 2, y=2) | E N D C _ P C 2 _ R 1 7 _ x L T E _ y N R |  | RP-211760 | Ericsson |
| 911111 | Core part: ENDC_PC2_R17_xLTE_yNR | E N D C _ P C 2 _ R 1 7 _ x L T E _ y N R - C o r e | R4 | RP-211760 | Ericsson |
| 911211 | Perf. Part: ENDC_PC2_R17_xLTE_yNR | E N D C _ P C 2 _ R 1 7 _ x L T E _ y N R - P e r f | R4 | RP-211760 | Ericsson |
| 930051 | UE Conformance - Power Class 2 for EN-DC with x LTE bands + y NR band(s) in DL and with 1 LTE band +1 TDD NR band in UL (either x= 2, 3, y=1 or x=1, 2, y=2) | E N D C _ P C 2 _ R 1 7 _ x L T E _ y N R - U E C o n T e s t | R5 | RP-211870 | Ericsson |
| 911018 | Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC | L T E _ N R _ S i m u l t _ R x T x |  | RP-211382 | Huawei |
| 911118 | Core part: LTE_NR_Simult_RxTx | L T E _ N R _ S i m u l t _ R x T x - C o r e | R4 | RP-211382 | Huawei |
| 911218 | Perf. part: LTE_NR_Simult_RxTx | L T E _ N R _ S i m u l t _ R x T x - P e r f | R4 | RP-211382 | Huawei |
| 890052 | Rel-17 Dual Connectivity (DC) of 5 bands LTE inter-band CA (5DL/1UL) and 1 NR band (1DL/1UL) | D C _ R 1 7 _ 5 B L T E _ 1 B N R _ 6 D L 2 U L |  | RP-211195 | Samsung |
| 890152 | Core part: DC_R17_5BLTE_1BNR_6DL2UL | D C _ R 1 7 _ 5 B L T E _ 1 B N R _ 6 D L 2 U L - C o r e | R4 | RP-211195 | Samsung |
| 890252 | Perf. part: DC_R17_5BLTE_1BNR_6DL2UL | D C _ R 1 7 _ 5 B L T E _ 1 B N R _ 6 D L 2 U L - P e r f | R4 | RP-211195 | Samsung |
| 890053 | Rel-17 Dual Connectivity (DC) of x bands (x=2, 3, 4) LTE inter-band CA (xDL/1UL) and 1 NR FR1 band (1DL/1UL) and 1 NR FR2 band (1DL/1UL) | D C _ R 1 7 _ x B L T E _ 2 B N R _ y D L 3 U L |  | RP-202542 | Samsung |
| 890153 | Core part: DC_R17_xBLTE_2BNR_yDL3UL | D C _ R 1 7 _ x B L T E _ 2 B N R _ y D L 3 U L - C o r e | R4 | RP-202542 | Samsung |
| 890253 | Perf. part: DC_R17_xBLTE_2BNR_yDL3UL | D C _ R 1 7 _ x B L T E _ 2 B N R _ y D L 3 U L - P e r f | R4 | RP-202542 | Samsung |
| 880080 | Band combinations for concurrent operation of NR/LTE Uu bands/band combinations and one NR/LTE V2X PC5 band | N R _ L T E _ V 2 X _ P C 5 _ c o m b o s |  | RP-210588 | CATT |
| 880180 | Core part: NR_LTE_V2X_PC5_combos | N R _ L T E _ V 2 X _ P C 5 _ c o m b o s - C o r e | R4 | RP-210588 | CATT |
| 880280 | Perf. part: NR_LTE_V2X_PC5_combos | N R _ L T E _ V 2 X _ P C 5 _ c o m b o s - P e r f | R4 | RP-210588 | CATT |
| 880094 | High power UE (power class 2) for EN-DC with 1 LTE band + 1 NR TDD band | E N D C _ U E _ P C 2 _ R 1 7 _ N R _ T D D |  | RP-211853 | China Unicom |
| 880194 | Core part: ENDC_UE_PC2_R17_NR_TDD | E N D C _ U E _ P C 2 _ R 1 7 _ N R _ T D D - C o r e | R4 | RP-211853 | China Unicom |
| 880294 | Perf. part: ENDC_UE_PC2_R17_NR_TDD | E N D C _ U E _ P C 2 _ R 1 7 _ N R _ T D D - P e r f | R4 | RP-211853 | China Unicom |
| 911000 | UE Conformance - ENDC_UE_PC2_R17_NR_TDD | E N D C _ U E _ P C 2 _ R 1 7 _ N R _ T D D - U E C o n T e s t | R5 | RP-211420 | China Unicom |
| 880095 | LTE/NR spectrum sharing in Band 38/n38 | D S S _ L T E _ B 3 8 _ N R _ B n 3 8 |  | RP-201314 | Vodafone |
| 880195 | Core part: DSS_LTE_B38_NR_Bn38 | D S S _ L T E _ B 3 8 _ N R _ B n 3 8 - C o r e | R4 | RP-201314 | Vodafone |
| 880096 | LTE/NR spectrum sharing in Band 40/n40 | D S S _ L T E _ B 4 0 _ N R _ B n 4 0 |  | RP-202084 | Reliance Jio |
| 880196 | Core part: DSS_LTE_B40_NR_Bn40 | D S S _ L T E _ B 4 0 _ N R _ B n 4 0 - C o r e | R4 | RP-202084 | Reliance Jio |
| 880296 | Perf. part: DSS_LTE_B40_NR_Bn40 | D S S _ L T E _ B 4 0 _ N R _ B n 4 0 - P e r f | R4 | RP-202084 | Reliance Jio |
| 880098 | Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) | D C _ R 1 7 _ 1 B L T E _ 1 B N R _ 2 D L 2 U L |  | RP-212096 | CHTTL |
| 880198 | Core part: DC_R17_1BLTE_1BNR_2DL2UL | D C _ R 1 7 _ 1 B L T E _ 1 B N R _ 2 D L 2 U L - C o r e | R4 | RP-212096 | CHTTL |
| 880298 | Perf. part: DC_R17_1BLTE_1BNR_2DL2UL | D C _ R 1 7 _ 1 B L T E _ 1 B N R _ 2 D L 2 U L - P e r f | R4 | RP-212096 | CHTTL |
| 880099 | Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL) | D C _ R 1 7 _ 2 B L T E _ 1 B N R _ 3 D L 2 U L |  | RP-212174 | Huawei |
| 880199 | Core part: DC_R17_2BLTE_1BNR_3DL2UL | D C _ R 1 7 _ 2 B L T E _ 1 B N R _ 3 D L 2 U L - C o r e | R4 | RP-212174 | Huawei |
| 890263 | Perf. part: DC_R17_2BLTE_1BNR_3DL2UL | D C _ R 1 7 _ 2 B L T E _ 1 B N R _ 3 D L 2 U L - P e r f | R4 | RP-212174 | Huawei |
| 881000 | Dual Connectivity (DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL) | D C _ R 1 7 _ 3 B L T E _ 1 B N R _ 4 D L 2 U L |  | RP-211758 | Ericsson |
| 881100 | Core part: DC_R17_3BLTE_1BNR_4DL2UL | D C _ R 1 7 _ 3 B L T E _ 1 B N R _ 4 D L 2 U L - C o r e | R4 | RP-211758 | Ericsson |
| 881200 | Perf. part: DC_R17_3BLTE_1BNR_4DL2UL | D C _ R 1 7 _ 3 B L T E _ 1 B N R _ 4 D L 2 U L - P e r f | R4 | RP-211758 | Ericsson |
| 881001 | Dual Connectivity (DC) of 4 bands LTE inter-band CA (4DL/1UL) and 1 NR band (1DL/1UL) | D C _ R 1 7 _ 4 B L T E _ 1 B N R _ 5 D L 2 U L |  | RP-212532 | Nokia |
| 881101 | Core part: DC_R17_4BLTE_1BNR_5DL2UL | D C _ R 1 7 _ 4 B L T E _ 1 B N R _ 5 D L 2 U L - C o r e | R4 | RP-212532 | Nokia |
| 881201 | Perf. part: DC_R17_4BLTE_1BNR_5DL2UL | D C _ R 1 7 _ 4 B L T E _ 1 B N R _ 5 D L 2 U L - P e r f | R4 | RP-212532 | Nokia |
| 881002 | Dual Connectivity (DC) of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) | D C _ R 1 7 _ x B L T E _ 2 B N R _ y D L 2 U L |  | RP-211750 | LG Electronics |
| 881102 | Core part: DC_R17_xBLTE_2BNR_yDL2UL | D C _ R 1 7 _ x B L T E _ 2 B N R _ y D L 2 U L - C o r e | R4 | RP-211750 | LG Electronics |
| 881202 | Perf. part: DC_R17_xBLTE_2BNR_yDL2UL | D C _ R 1 7 _ x B L T E _ 2 B N R _ y D L 2 U L - P e r f | R4 | RP-211750 | LG Electronics |
| 881003 | Dual Connectivity (DC) of x bands (x=1,2) LTE inter-band CA (xDL/xUL) and y bands (y=3-x) NR inter-band CA (yDL/yUL) | D C _ R 1 7 _ x B L T E _ y B N R _ 3 D L 3 U L |  | RP-211803 | ZTE |
| 881103 | Core part: DC_R17_xBLTE_yBNR_3DL3UL | D C _ R 1 7 _ x B L T E _ y B N R _ 3 D L 3 U L - C o r e | R4 | RP-211803 | ZTE |
| 881203 | Perf. part: DC_R17_xBLTE_yBNR_3DL3UL | D C _ R 1 7 _ x B L T E _ y B N R _ 3 D L 3 U L - P e r f | R4 | RP-211803 | ZTE |
| 881004 | Dual Connectivity (DC) of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 3 bands NR inter-band CA (3DL/1UL) | D C _ R 1 7 _ x B L T E _ 3 B N R _ y D L 2 U L |  | RP-211805 | ZTE |
| 881104 | Core part: DC_R17_xBLTE_3BNR_yDL2UL | D C _ R 1 7 _ x B L T E _ 3 B N R _ y D L 2 U L - C o r e | R4 | RP-211805 | ZTE |
| 881204 | Perf. part: DC_R17_xBLTE_3BNR_yDL2UL | D C _ R 1 7 _ x B L T E _ 3 B N R _ y D L 2 U L - P e r f | R4 | RP-211805 | ZTE |
| 881012 | band combinations for SA NR supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP) | N R _ S U L _ c o m b o s _ R 1 7 |  | RP-212178 | Huawei |
| 881112 | Core part: NR_SUL_combos_R17 | N R _ S U L _ c o m b o s _ R 1 7 - C o r e | R4 | RP-212178 | Huawei |
| 881212 | Perf. part: NR_SUL_combos_R17 | N R _ S U L _ c o m b o s _ R 1 7 - P e r f | R4 | RP-212178 | Huawei |
| 900056 | UE Conformance - Rel-17 NR CA and DC; and NR and LTE DC Configurations | N R _ C A D C _ N R _ L T E _ D C _ R 1 7 - U E C o n T e s t | R5 | RP-211357 | Huawei |
| 0 | Rel-17 NR new/modified bands | - |  | - | - |
| 911016 | Introduction of 900MHz NR band for Europe for Rail Mobile Radio (RMR) | N R _ R A I L _ E U _ 9 0 0 M H z |  | RP-211495 | UIC |
| 911116 | Core part: NR_RAIL_EU_900MHz | N R _ R A I L _ E U _ 9 0 0 M H z - C o r e | R4 | RP-211495 | UIC |
| 911216 | Perf. part: NR_RAIL_EU_900MHz | N R _ R A I L _ E U _ 9 0 0 M H z - P e r f | R4 | RP-211495 | UIC |
| 911017 | Introduction of 1900MHz NR TDD band for Europe for Rail Mobile Radio (RMR) | N R _ R A I L _ E U _ 1 9 0 0 M H z _ T D D |  | RP-211542 | UIC |
| 911117 | Core part: NR_RAIL_EU_1900MHz_TDD | N R _ R A I L _ E U _ 1 9 0 0 M H z _ T D D - C o r e | R4 | RP-211542 | UIC |
| 911217 | Perf. part: NR_RAIL_EU_1900MHz_TDD | N R _ R A I L _ E U _ 1 9 0 0 M H z _ T D D - P e r f | R4 | RP-211542 | UIC |
| 860041 | Extending current NR operation to 71GHz | N R _ e x t _ t o _ 7 1 G H z | R1 | RP-212637 | Qualcomm |
| 860141 | Core part: Extending current NR operation to 71GHz | N R _ e x t _ t o _ 7 1 G H z - C o r e | R1 | RP-212637 | Qualcomm |
| 860241 | Perf. part: Extending current NR operation to 71GHz | N R _ e x t _ t o _ 7 1 G H z - P e r f | R4 | RP-212637 | Qualcomm |
| 860037 | Study on supporting NR from 52.6 GHz to 71 GHz | F S _ N R _ 5 2 _ t o _ 7 1 G H z | R1 | RP-201838 | Intel |
| 870060 | Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz | F S _ N R _ I M T _ p a r a m | R4 | RP-202276 | Ericsson |
| 870063 | Introduction of NR band n13 | N R _ n 1 3 | R4 | RP-200480 | Huawei |
| 870163 | Core part: Introduction of NR band n13 | N R _ n 1 3 - C o r e | R4 | RP-200480 | Huawei |
| 870263 | Perf. part: Introduction of NR band n13 | N R _ n 1 3 - P e r f | R4 | RP-200480 | Huawei |
| 880082 | Introduction of NR 47GHz band | N R _ 4 7 G H z _ b a n d | R4 | RP-212528 | Ericsson |
| 880182 | Core part: Introduction of NR 47GHz band | N R _ 4 7 G H z _ b a n d - C o r e | R4 | RP-212528 | Ericsson |
| 880282 | Perf. part: Introduction of NR 47GHz band | N R _ 4 7 G H z _ b a n d - P e r f | R4 | RP-212528 | Ericsson |
| 880084 | Introduction of NR band 24 | N R _ b a n d _ n 2 4 | R4 | RP-211505 | Ligado Networks |
| 880184 | Core part: Introduction of NR band 24 | N R _ b a n d _ n 2 4 - C o r e | R4 | RP-211505 | Ligado Networks |
| 880284 | Perf. part: Introduction of NR band 24 | N R _ b a n d _ n 2 4 - P e r f | R4 | RP-211505 | Ligado Networks |
| 890050 | Introduction of 6GHz NR licensed bands | N R _ 6 G H z | R4 | RP-202844 | Huawei |
| 890150 | Core part: Introduction of 6GHz NR licensed bands | N R _ 6 G H z - C o r e | R4 | RP-202844 | Huawei |
| 890250 | Perf. part: Introduction of 6GHz NR licensed bands | N R _ 6 G H z - P e r f | R4 | RP-202844 | Huawei |
| 920075 | Introduction of operation in full unlicensed band 5925-7125MHz for NR | N R _ 6 G H z _ u n l i c _ f u l l | R4 | RP-212302 | Apple |
| 920175 | Core part: Introduction of operation in full unlicensed band 5925-7125MHz for NR | N R _ 6 G H z _ u n l i c _ f u l l - C o r e | R4 | RP-212302 | Apple |
| 920275 | Perf. part: Introduction of operation in full unlicensed band 5925-7125MHz for NR | N R _ 6 G H z _ u n l i c _ f u l l - P e r f | R4 | RP-212302 | Apple |
| 890051 | Introduction of lower 6GHz NR unlicensed operation for Europe | N R _ 6 G H z _ u n l i c _ E U | R4 | RP-212625 | Nokia |
| 890151 | Core part: Introduction of lower 6GHz NR unlicensed operation for Europe | N R _ 6 G H z _ u n l i c _ E U - C o r e | R4 | RP-212625 | Nokia |
| 890251 | Perf. part: Introduction of lower 6GHz NR unlicensed operation for Europe | N R _ 6 G H z _ u n l i c _ E U - P e r f | R4 | RP-212625 | Nokia |
| 900065 | Introduction of NR band n67 | N R _ n 6 7 | R4 | RP-202829 | Ericsson |
| 900165 | Core part: Introduction of NR band n67 | N R _ n 6 7 - C o r e | R4 | RP-202829 | Ericsson |
| 900265 | Perf. Part: Introduction of NR band n67 | N R _ n 6 7 - P e r f | R4 | RP-202829 | Ericsson |
| 900066 | Introduction of NR band n85 | N R _ n 8 5 | R4 | RP-210707 | Ericsson |
| 900166 | Core part: Introduction of NR band n85 | N R _ n 8 5 - C o r e | R4 | RP-210707 | Ericsson |
| 900266 | Perf. part: Introduction of NR band n85 | N R _ n 8 5 - P e r f | R4 | RP-210707 | Ericsson |
| 880083 | Introduction of 1.6 GHz NR supplemental uplink (SUL) band with same uplink frequency range of Band 24 | N R _ S U L _ U L _ n 2 4 |  | RP-210341 | Ligado Networks |
| 880183 | Core part: NR_SUL_UL_n24 | N R _ S U L _ U L _ n 2 4 - C o r e | R4 | RP-210341 | Ligado Networks |
| 880283 | Perf. part: NR_SUL_UL_n24 | N R _ S U L _ U L _ n 2 4 - P e r f | R4 | RP-210341 | Ligado Networks |
| 880085 | Introduction of NR supplemental uplink (SUL) band 1880-1920MHz | N R _ S U L _ b a n d _ 1 8 8 0 _ 1 9 2 0 M H z |  | RP-201363 | CMCC |
| 880185 | Core part: NR_SUL_band_1880_1920MHz | N R _ S U L _ b a n d _ 1 8 8 0 _ 1 9 2 0 M H z - C o r e | R4 | RP-201363 | CMCC |
| 880285 | Perf. part: NR_SUL_band_1880_1920MHz | N R _ S U L _ b a n d _ 1 8 8 0 _ 1 9 2 0 M H z - P e r f | R4 | RP-201363 | CMCC |
| 880086 | Introduction of NR supplemental uplink (SUL) band 2300-2400MHz | N R _ S U L _ b a n d _ 2 3 0 0 _ 2 4 0 0 M H z |  | RP-201364 | CMCC |
| 880186 | Core part: NR_SUL_band_2300_2400MHz | N R _ S U L _ b a n d _ 2 3 0 0 _ 2 4 0 0 M H z - C o r e | R4 | RP-201364 | CMCC |
| 880286 | Perf. part: NR_SUL_band_2300_2400MHz | N R _ S U L _ b a n d _ 2 3 0 0 _ 2 4 0 0 M H z - P e r f | R4 | RP-201364 | CMCC |
| 900055 | UE Conformance - New Rel-17 NR licensed bands and extension of existing NR bands | N R _ l i c _ b a n d s _ B W _ R 1 7 - U E C o n T e s t | R5 | RP-202567 | Huawei |
| 0 | Rel-17 LTE new/modified bands | - |  | - | - |
| 911020 | New bands and bandwidth allocation for 5G terrestrial broadcast - part 1 | L T E _ t e r r _ b c a s t _ b a n d s _ p a r t 1 | R1 | RP-211144 | EBU |
| 911120 | New bands and bandwidth allocation for 5G terrestrial broadcast - part 1 | L T E _ t e r r _ b c a s t _ b a n d s _ p a r t 1 | R1 | RP-211144 | EBU |
| 880079 | Additional LTE bands for UE categories M1/M2/NB1/NB2 | L T E _ b a n d s _ R 1 7 _ M 1 _ M 2 _ N B 1 _ N B 2 |  | RP-211857 | Ericsson |
| 880179 | Core part: LTE_bands_R17_M1_M2_NB1_NB2 | L T E _ b a n d s _ R 1 7 _ M 1 _ M 2 _ N B 1 _ N B 2 - C o r e | R4 | RP-211857 | Ericsson |
| 880279 | Perf. part: LTE_bands_R17_M1_M2_NB1_NB2 | L T E _ b a n d s _ R 1 7 _ M 1 _ M 2 _ N B 1 _ N B 2 - P e r f | R4 | RP-211857 | Ericsson |
| 880081 | Modification of LTE Band 24 Specifications to comply with updated regulatory emission limits | L T E _ B 2 4 _ m o d |  | RP-211507 | Ligado Networks |
| 880181 | Core part: LTE_B24_mod | L T E _ B 2 4 _ m o d - C o r e | R4 | RP-211507 | Ligado Networks |
| 880281 | Perf. part: LTE_B24_mod | L T E _ B 2 4 _ m o d - P e r f | R4 | RP-211507 | Ligado Networks |
| 920067 | UE Conformance - Modification of LTE Band 24 Specifications to comply with updated regulatory emission limits | L T E _ B 2 4 _ m o d - U E C o n T e s t | R5 | RP-212497 | Ligado Networks |
| 0 | Rel-17 LTE inter-band Carrier Aggregation (LTE iterbCA) | - |  | - | - |
| 880087 | LTE iterbCA for 2 bands DL with 1 band UL | L T E _ C A _ R 1 7 _ 2 B D L _ 1 B U L |  | RP-211859 | Qualcomm |
| 880187 | Core part: LTE_CA_R17_2BDL_1BUL | L T E _ C A _ R 1 7 _ 2 B D L _ 1 B U L - C o r e | R4 | RP-211859 | Qualcomm |
| 880287 | Perf. part: LTE_CA_R17_2BDL_1BUL | L T E _ C A _ R 1 7 _ 2 B D L _ 1 B U L - P e r f | R4 | RP-211859 | Qualcomm |
| 880088 | LTE iterbCA for 3 bands DL with 1 band UL | L T E _ C A _ R 1 7 _ 3 B D L _ 1 B U L |  | RP-212192 | Huawei |
| 880188 | Core part: LTE_CA_R17_3BDL_1BUL | L T E _ C A _ R 1 7 _ 3 B D L _ 1 B U L - C o r e | R4 | RP-212192 | Huawei |
| 880288 | Stopped - Perf. part: LTE iterbCA for 3 bands DL with 1 band UL | L T E _ C A _ R 1 7 _ 3 B D L _ 1 B U L - P e r f | R4 | RP-202615 | Huawei |
| 880089 | LTE iterbCA Core for x bands DL (x=4, 5, 6) with 1 band UL | L T E _ C A _ R 1 7 _ x B D L _ 1 B U L |  | RP-211885 | Nokia |
| 880189 | Core part: LTE_CA_R17_xBDL_1BUL | L T E _ C A _ R 1 7 _ x B D L _ 1 B U L - C o r e | R4 | RP-211885 | Nokia |
| 880289 | Perf. part: LTE_CA_R17_xBDL_1BUL | L T E _ C A _ R 1 7 _ x B D L _ 1 B U L - P e r f | R4 | RP-211885 | Nokia |
| 880090 | LTE iterbCA for 2 bands DL with 2 bands UL | L T E _ C A _ R 1 7 _ 2 B D L _ 2 B U L |  | RP-212194 | Huawei |
| 880190 | Core part: LTE_CA_R17_2BDL_2BUL | L T E _ C A _ R 1 7 _ 2 B D L _ 2 B U L - C o r e | R4 | RP-212194 | Huawei |
| 880290 | Perf. part: LTE_CA_R17_2BDL_2BUL | L T E _ C A _ R 1 7 _ 2 B D L _ 2 B U L - P e r f | R4 | RP-212194 | Huawei |
| 880091 | LTE iterbCA for x bands DL (x= 3, 4, 5) with 2 bands UL | L T E _ C A _ R 1 7 _ x B D L _ 2 B U L |  | RP-211748 | LG |
| 880191 | Core part: LTE_CA_R17_xBDL_2BUL | L T E _ C A _ R 1 7 _ x B D L _ 2 B U L - C o r e | R4 | RP-211748 | LG |
| 880291 | Perf. part: LTE_CA_R17_xBDL_2BUL | L T E _ C A _ R 1 7 _ x B D L _ 2 B U L - P e r f | R4 | RP-211748 | LG |

Annex A:
Template for contributing

Editor's note: To contribute to this document, please copy-paste the following template into a document to be submitted to the TSG plenary as "Work Item Summary". This will be further endorsed by the plenary.
Each summary is expected to be one page long maximum. 
The target audience of this document are readers inside and outside the 3GPP community. Contributions should be written as to be understandable by non-specialist. 
The summary should reflect the actual outcome of the Feature - and not the initial intentions.

---

Summary based on the input provided by [company name] in [tdoc numbed].

| Unique_ID | Name | Acronym | WG | WID | WI_rapporteur_name |
| --- | --- | --- | --- | --- | --- |
| AABBCC | Title here | XXX | e.g. S4 | xP-19XXXX | Firstname Familyname, Company |

Introduction

<guidance:
- summarize in one to a few sentences what was achieved by this WI and how: what problems it solves, what new functionality it adds, etc.;

- remember that only what ended up in the specs is of interest;

- clarify what is the relation of this WI to other WIs (e.g. is this an enhancement of a former WI? Is there a related WI?) and what was the status before this WI

Description

<guidance:

- Explain here in a couple of paragraphs the main impacts on the system, mostly: the new and/or impacted entities, and the big lines of the system's behaviour;

- remember that only what ended up in the specs shall be covered here;

- this it not intended to be a copy and paste from the specifications, the WID or status report;

- the reader shall understand how the key functionalities are working without running too far into details;

- use figures, tables, references to illlustrate and  facilitate the understanding;

- remember that you do not write the WI summary for people who contributed to this WI so make sure that you use terminology and abbreviations that can be understood by any engineer or make sure you explain them;

- When the usual Stage 1/Stage 2/Stage 3 process is not followed -and in particular if a full stage is missing- explain why.

- make the following test: give this summary to a colleague who was not at all involved in this topic and let this colleague explain in her/his own words how your WI works after reading the summary>

References

<guidance: It is recommended to add some references where the interested reader can find further documentation, e.g.

- new/impacted TRs and TSs (if any)

- last status report (if any)

- main CRs

NOTE: Please provide references that can be found easily, e.g. indicating Tdoc numbers.>

[1]	TS XX.XXX, Spec title

[2]	TS YY.YYY, Spec title

Annex B:
Process to get further information

# B.1	General

Since the present document is limited to provide an overview of each Feature, this chapter explains how to get additional information, in particular how to retrieve all the Specifications (TSs) and Reports (TRs) as well as all the CRs which relate to a given Work Item.

The Unique Identifier (UID) is the key to get additional information on a given Work Item. It can be found in the table located just below the clause's header. The table has the following format:

Table B-1: table format

| Unique Identifier (UID) | Name | Acronym | Outline Level (1=Feature, 2=Building Block, 3=Work Task) | Responsible Working Group | Work Item Description |
| --- | --- | --- | --- | --- | --- |

For readability reasons, the table headers are omitted in the continuation of the present document.

For instance, for the "Mission Critical Push to Talk over LTE Realignment", the table has to be understood as:

Table B-2: Example of table at the introduction of each Feature

| Unique Identifier (UID) | Name | Acronym | Outline Level | Responsible Working Group | Work Item Description |
| --- | --- | --- | --- | --- | --- |
| 700029 | Mission Critical Push to Talk over LTE Realignment | MCImp-MCPTTR | 2 | S1 | SP-150821 |

Thus, the UID for this Work Item is 700029.

Two methods are now possible to retrieve more information on a given feature: the "Step by step method" and the "Direct method". The "direct method" is faster but implies to know the hierarchical structure of the Work Items. The "step by step method" is slower but is easier to use, in particular when the hierarchical structure is unknown.

For instance, for retrieving all the CRs that relate to "Enhancements for Mission Critical Push To Talk", the search has to be done on UID 740022 but also potentially on its children Work Items (UID 720056, 740023 and 740024).

Table B-3: Example of a hierarchical structure and its consequences on the search procedure

| 740022 | Enhancements for Mission Critical Push To Talk | MCImp-eMCPTT |  | SP-160490 |
| --- | --- | --- | --- | --- |
| 720056 | Stage 2 of Enhancements for Mission Critical Push To Talk | MCImp-eMCPTT | S6 | SP-160490 |
| 740023 | Stage 3 of Enhancements for Mission Critical Push To Talk | MCImp-eMCPTT-CT | CT | CP-160824 |
| 740024 | CT1 aspects of Enhancements for Mission Critical Push To Talk | MCImp-eMCPTT-CT | C1 | CP-160824 |

These two methods are described in the following clauses.

# B.2	Direct method

The links below lead to the pages containing respectively all the Specifications and all the Change Requests (CRs) linked to a given Feature:

https://portal.3gpp.org/Specifications.aspx?q=1&WiUid=[UID]

https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=[UID]

where "[UID]" has to be preplaced by the UID value.

Using the example provided in the table 4.1-3, the specification linked to "Stage 2 of Enhancements for Mission Critical Push To Talk" can be found in:

https://portal.3gpp.org/Specifications.aspx?q=1&WiUid=720056

And all the related Change Requests are listed in:

https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=720056

# B.3	Step by step method

This method is to be used when the hierarchical structure is not known or when the "direct method" above does not show the expected results.

In this case, the 3GPP Ultimate web site has to be used:

https://portal.3gpp.org

As a preliminary step, it is essential that the "Customized Selection" is set to "All TSGs" (otherwise, a filter would be applied).

Figure B.3-1: Selecting "All TSGs" in "Customized Selection" as to remove any potential filter on the Search

Then select the "Work Plan" tab (upper red arrow in the figure below).

Figure B.3-2: Using the 3GU Portal to retrieve more information on a given Feature

Then the search might be performed by either typing the Acronym (as shown by the left red arrow on the figure above, using the example "MCPTT"), or by the name or UID (right box) then by clicking on the "Search" button. Watch the "Granularity (Level)" field, which is a filter to return only the Items which level is specified here.

In the results, the icon depicting some binoculars has to be hit (lower right red arrow on the figure above).

This will lead to the page shown in the figure below:

Figure B.3-3: Window resulting from a "Search"

On this window, the "Related" tab has to be clicked, as pointed by the red arrow in the figure above. This will lead to the window depicted in the figure below.

Figure B.3-4: "Related" tab in a Work Item search, with links to all related Specifications and Change Requests

The two links pointed by the red arrows in the figure above lead to the pages containing respectively all the Specifications and all the Change Requests (CRs) linked to this Feature.

Annex C:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-11 | TSG#94 |  |  |  |  | Initital draft, call for contributions | 0.1.0 |
| 2022-01 | - |  |  |  |  | Clean-up of skeletton | 0.2.0 |
| 2022-03 | TSG#95e |  |  |  |  | Incorporation of: SP-220019 WI Summary for Mission critical security enhancements phase 2 (MCXSec2) Motorola Solutions Danmark A/S SP-220020 Summary for Edge Computing Management Samsung R&D Institute UK SP-220075 Summary for Network Slice charging based on 5G Data Connectivity MATRIXX Software SP-220089 EMCData3 WI Summary AT&T GNS Belgium SPRL SP-220260 Summary for Enhancement for the 5G Control Plane Steering of Roaming for UE in CONNECTED mode (eCPSOR_CON) NTT DOCOMO INC. SP-220261 WI Summary for TEI17_SPSFAS China Telecommunications SP-220275 Summary for WI Support of Immersive Teleconferencing and Telepresence for Remote Terminals (ITT4RT) Nokia Corporation (Rapporteur) SP-220277 Summary for eV2XARC_Ph2 LG Electronics SP-220285 Summary for 5GMARCH China Mobile Com. Corporation SP-220289 Summary for Authentication and key management for applications based on 3GPP credential in 5G China Mobile CP-220021 Template for Feature Summary for inclusion in TR 21.917 (Rel-17 Description) MCC/Alain CP-220110 Summary for Stage 3 aspects of eMCCI FirstNet CP-220111 Summary for Stage 3 aspects of enh3MCPTT FirstNet CP-220149 Summary for Restoration of PDN Connections in PGW-C/SMF Set Nokia, Nokia Shanghai Bell CP-220150 Summary for Non-Seamless WLAN offload authentication in 5GS Nokia, Nokia Shanghai Bell CP-220152 Summary for Enhancement of Inter-PLMN Roaming Huawei CP-220211 Summary for Start of Pause of Charging via User Plane Ericsson CP-220212 Summary for Enhancement on the GTP-U entity restart Ericsson CP-220321 Summary for PFD management enhancement Huawei CP-220322 Summary for enhancement of 5G PCC related services in Rel-17 Huawei CP-220343 Summary for Restoration of Profiles related to UDR NTT DOCOMO INC. CP-220347 Summary for Rel-17 Enhancements of 3GPP Northbound Interfaces and Application Layer APIs Huawei CP-220368 Summary for PAP/CHAP protocols usage in 5GS China Telecomunication Corp. | 0.3.0 |
|  |  |  |  |  |  | Incorporation of: RP-220142 WI summary for WI NR Uplink Data Compression (UDC) CATT RP-220154 WI summary for WI NR small data transmissions in INACTIVE state (Rel-17) ZTE Corporation (rapporteur) RP-220165 Summary for SI Study on band combination handling in RAN4 ZTE Corporation RP-220191 WI summary for WI Introduction of DL 1024QAM for NR FR1; rapporteur: Ericsson, Nokia Ericsson RP-220211 WI summary for WI NR Sidelink Relay OPPO RP-220316 WI summary for NR support for high speed train scenario in frequency range 2 (FR2) Samsung RP-220408 WI summary for WI NR Multicast and Broadcast Services Huawei, HiSilicon RP-220410 WI summary for WI Multi-Radio Dual-Connectivity enhancements Huawei, HiSilicon RP-220445 WI summary for WI New bands and bandwidth allocation for 5G terrestrial broadcast (part 1) EBU, Qualcomm Incorporated (Rapporteur) RP-220464 WI summary for Rel17 WI on NR Dynamic spectrum sharing (DSS) Ericsson RP-220521 WI summary for WI NR sidelink enhancement LG Electronics RP-220530 WI summary for WI Additional enhancements for NB-IoT and LTE-MTC Huawei, HiSilicon RP-220544 WI summary for WI NR Repeaters Qualcomm Incorporated RP-220556 WI summary for WI NR QoE management and optimizations for diverse services China Unicom RP-220560 WI summary for WI Enhanced eNB(s) architecture evolution for E-UTRAN and NG-RAN China Unicom RP-220562 WI summary for WI Enhancement of Private Network Support for NG-RAN China Telecom RP-220564 WI summary for WI NR coverage enhancements China Telecom RP-220604 WI summary for WI Core part: Support for Multi-SIM devices for LTE/NR vivo (WI Rapporteur) RP-220606 WI summary for WI: Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC) vivo (WI Rapporteur) RP-220631 WI summary for WI: Enhanced NR support for high speed train scenario for frequency range 1 (FR1) CMCC RP-220682 WI summary for WI: RF requirements enhancement for NR frequency range 1 (FR1) Huawei, HiSilicon RP-220752 WI summary for WI NR and MR-DC measurement gap enhancements Rapporteur (MediaTek Inc., Intel) RP-220802 WI summary for WI Core part: Further enhancements on MIMO for NR Samsung RP-220822 WI summary for WI Enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) CMCC RP-220919 WI summary for WI Core part: NR positioning enhancements Intel RP-220923 WI summary for WI UE RF requirements for Transparent Tx Diversity (TxD) for NR Qualcomm Incorporated | 0.4.0 |

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=

[1]	TS 33.180: “Security of the Mission Critical (MC) service; (Release 17)”