| 3GPP TR 23.700-35 V1.1.0 (2025-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Service Enabler Architecture Layer (SEAL) Phase 4; (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	SEAL services technical gap analysis	9

4.1	Technical gap#1: NRM service	9

4.1.1	Analysis	9

4.1.2	gaps	10

4.2	Technical gap #2: SEALDD service	10

4.2.1	Analysis	10

4.2.2	Identified gaps	10

4.3	Technical Gap#3: Direct and/or indirect interaction \with CN NFs per different deployments and requirements	10

4.3.1	Analysis	10

4.3.2	Gaps	10

4.4	Technical gap #4: NRM service to support IoT	11

4.4.1	Analysis	11

4.4.2	Identified gaps	12

4.5	Technical gap #5: Security Credential	12

4.5.1	Analysis	12

4.5.2	Identified gaps	14

4.6.	Technical Gap#6: SEAL server API consumers	14

4.6.1	Analysis	14

4.6.2	Identified gaps	14

4.7	Technical gap#7: Distinguishing stage 2 SEAL services design with stage 3 API definition	14

4.7.1	Analysis	14

4.7.2	Identified gaps	14

4.8   Technical gap #8: API design guidelines for SEAL services	14

4.8.1	Analysis	14

4.8.2	Identified gaps	15

4.x.	<Existing SEAL service #X>	15

4.x.1	Analysis	15

4.x.2	Identified gaps	15

5	SEAL services adoption gap analysis	15

5.1	Generic gap #1: Representention of SEAL services within 3GPP system	15

5.1.1	Analysis	15

5.1.2	Identified gaps	15

5.2	Void	16

5.3	Adoption gap #2: Representation of SEAL services within the whole 3GPP network exposure system	16

5.3.1	Analysis	16

5.3.2	Identified gaps	16

5.4	Adoption gap #3: Linking SEAL services to target application use cases	16

5.4.1	Analysis	16

5.4.2	Identified gaps	16

5.5	Adoption Gap#5: Incomplete business relationship for SEAL service	16

5.5.1	Analysis	16

5.5.2	Identified gaps	17

5.6	Adoption Gap#6: Deployment options involving SEAL Client	17

5.6.1	Analysis	17

5.6.2	Identified gaps	17

5.7	Generic gap#7: The adoption of SEAL layer by External Organizations	17

5.7.1	Analysis	17

5.7.2	Identified gaps	17

6	Void	17

7	Architecture requirements	17

7.1	< API design guidance> requirements	17

8	Solutions	18

8.0	Solutions and gaps mapping	18

8.1	Solutions for SEAL services technical gaps	19

8.1.1	Technical solution#1 SEALDD APIs improvement	19

8.1.2	Technical solution #2: < SEAL services and SEAL API design guidelines>	23

8.1.3	Technical solution#3: SEAL service design guidance framework	25

8.1.4	Technical solution 4: The specification changes for direct interaction between LMS and GMLC/LMF	26

8.1.5	Technical Solution#5: Align Security Credential with 3GPP TS 33.434 [13] and Clean Application Enablement Specifications	36

8.1.6	Technical Solution #6: Multiple NRM services for network service QoS control	36

8.1.7	Solution#7: Enhancements of NRM network resource adaptation service	48

8.1.8	Technical solution #8: Representation of SEAL service consumer	50

8.1.9	Technical solution #9: New NRM service for IoT power saving	51

8.1.10	Technical Solution # 10: Single NRM service for network service QoS control for applications	53

8.1.11	Solution #11: New SEAL service for power saving in IoT devices	57

8.1.12	Technical solution 12: Solution for stage 2 services design in SEAL specifications	60

8.1.13	Technical solution#13: Service design guidelines for common functionalities	63

8.2	Solutions for SEAL services adoption gaps	64

8.2.1	Adoption Solution#1: Service based Architecture of 3GPP system supporting SEAL services	64

8.2.2	Adoption Solution#2: Layered representation of 3GPP network exposure system	66

8.2.3	Adoption solution#3: business relationship involving SEAL Client provider	69

8.2.4	Adoption Solution # 4: SEAL deployment involving SEAL Client(s)	71

8.2.5	Adoption Solution#5: Target application use case of SEAL services	73

8.2.6	Adoption Solution #6: Alternative solutions for the SEAL mapping to an external SDO‘s system	81

9	Overall evaluation	83

9.1	Solution evaluations for technical gaps	84

9.1.1	Overall evaluation on technical gap #1	84

9.1.2	Overall evaluation on technical gap #2	84

9.1.3	Overall evaluation on technical gap #3	84

9.1.4	Overall evaluation on technical gap #4	85

9.1.5	Overall evaluation on technical gap #5	85

9.1.6	Overall evaluation on technical gap #6	85

9.1.7	Overall evaluation on technical gap #7	85

9.1.8	Overall evaluation on technical gap #8	85

9.2	Gap evaluation for adoption gaps	86

9.2.1	Overall evaluation on adoption gap#1	86

9.2.2	Overall evaluation on adoption gap#2	86

9.2.3	Overall evaluation on adoption gap#3	86

9.2.5	Overall evaluation on adoption gap#5	86

9.2.6	Overall evaluation on adoption gap#6	86

9.2.7	Overall evaluation on adoption gap#7	86

10	Conclusions	87

10.1	General conclusions	87

10.2	Conclusions of SEAL services technical gap #x	87

10.3	Conclusions of SEAL services adoption gaps	87

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e., technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In drafting the TS/TR, pay particular attention to the use of modal auxiliary verbs! TRs shall not contain any normative provisions.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The scope of Technical Report is to study on potential enhancements to Service Enabler Architecture Layer (SEAL). In particular, the study reviews the existing SEAL specifications and architecture to identify any gaps or potential enhancements necessary to enable usage of SEAL for 3rd party applications, including aspects:

- Representation of SEAL as part of 3GPP ecosystem and customer’s ecosystem.

- Additional deployment options considering the SEAL client(s).

- Use cases and advantages of adopting SEAL services.

- Providing design guidelines that need to be adhered for developing new SEAL services.

- Iimproving existing SEAL services to make it easier to be consumed.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications; Stage2".

[3]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Stage 2".

[4]	3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service; Stage 2".

[5]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service; Stage 2".

[6]	3GPP TS 23.437: "Service Enabler Architecture Layer for Verticals (SEAL); Spatial mapping and Spatial anchors management Stage 2".

[7]	3GPP TS 23.438: "Service Enabler Architecture Layer for Verticals (SEAL); Digital asset".

[8]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[9]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2".

[10]	3GPP TS 23.501: "System architecture for the 5G System (5GS)" 2".

[11]	3GPP TS 23.502: "Procedures for the 5G System (5GS)"

[12]	3GPP TS 23.482: "Functional architecture and information flows for AIML Enablement Service".

[13]	3GPP TS 33.434: "Security aspects of Service Enabler Architecture Layer (SEAL) for verticals".

[14]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[15]	3GPP TS 29.122: " T8 reference point for Northbound APIs ".

[16]           3GPP TR 22.856: "Feasibility Study on Localized Mobile Metaverse Services".

[17]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[18]	3GPP TS 22.186: "Enhancement of 3GPP support for V2X scenarios".

[19]	3GPP TS 28.538: " Edge Computing Management".

[20]	3GPP TS 23.558: " Architecture for enabling Edge Applications".

[21]           3GPP TS 23.286: "Application layer support for Vehicle-to-Everything (V2X) services Functional architecture and information flows".

[22]           3GPP TS 23.255: "Application layer support for Uncrewed Aerial System (UAS); Functional architecture and information flows".

[23]           3GPP TS 23.239: "Application enablement aspects for MMTel".

[24]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS)".

[25]	3GPP TS 38.331: " NR; Radio Resource Control (RRC) protocol specification".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	SEAL services technical gap analysis

## 4.1	Technical gap#1: NRM service

### 4.1.1	Analysis

XR is an emerging mobile media service expected to contribute more and more traffics to 5G network. 3GPP specifies several new capabilities to support this feature, including the enhancement of PCF/N33 APIs towards the AF/AS. There are some gaps from the easy-to-use perspective.

(1)	Unnecessary parameters for XR, which is against minimum parameters principle

Currently the PCF/N33 APIs are common to all the verticals and its input parameters for the PCF/N33 APIs are increasing more and more and become excessive complicated. From the XR service point of view, some parameters introduced by URLLC is not required. Those non-XR related parameters, may cause confusion and mis-use risks for XR application developers.

(2) API parameters is not application developers understandable. XR specific input parameters covers several in the PCF/N33 API e.g., AFSessionWithQoS are present for various purposes (or features as defined in 3GPP TS 29.522).

The following table show the 5G added features to AsSessionWithQoS.  All the yellow highlighted features can be used by eXtended Reality (XR) and interactive media services, but not been described. It is difficult for XR application developers to link them with certain XR related service.

| AsSessionWithQoS | - The following 5G-only features defined in clause 5.14.4 of 3GPP TS 29.122 [4] may be supported only by the NEF: "EthAsSessionQoS_5G", "QoSMonitoring_5G", "PacketDelayFailureReport", "MacAddressRange_5G", "AlternativeQoS_5G", "TSC_5G", "DisableUENotification_5G", "ExposureToEAS", "AltQosWithIndParams_5G", "EnEthAsSessionQoS_5G", "enNB_5G", "AltQoSProfilesSupportReport", "ExtQoS_5G", "EnTSCAC", "L4S", "MultiMedia", "PowerSaving", "EnQoSMon", "PDUSetHandling", "RTLatency", "ToSTC_5G", "QoSTiming_5G", "ListUE_5G" and "GMEC". and "QoSMonCapRepo". - The "LOSS_OF_BEARER", "RECOVERY_OF_BEARER" and "RELEASE_OF_BEARER" events do not apply for 5G. |
| --- | --- |

NRM are defined to ease the usage of 5G network resource services, but there are some issues to well support the XR service:

-	The existing input parameters of the SS_NetworkResourceAdaptation API are not sufficient to support the latest network XR capabilities to be used by the XR service, e.g., lack of the support of PDU set handling.

-	The issue of unnecessary parameters for XR also exists, i.e., SS_NetworkResourceAdaptation API cover all the verticals XR, TSN, TSC and so on, which make it is not friendly for the XR service to identify which parameters to be used.

The above issues are caused due to the lack of application - oriented and friendly API design principle including:

-	The parameters and granularity of service shall be defined in application developer’s understandable way.

-	Minimum parameters for an API to support certain service.

Similarly, for other applications e.g., multi-modal communications, cyber-physical control applications, V2X, rail communications, AV production service, medical or gaming applications may have the same issue to use the existing NEF API and NRM API to request the correct network resource and handling for their traffic.

So how to design NRM services complied with above principle needs to be studied.

### 4.1.2	gaps

-Gap 4-1-2-1: How to enhance NRM services API for various services/applications to ease the usage of 5G services by the application developer.

## 4.2	Technical gap #2: SEALDD service

### 4.2.1	Analysis

Currently, the services defined in SEALDD can be improved to expose a better service to the 3rd party. The improvement can contain following aspect:

The functions and values supported by services defined in SEALDD need to be further improved to make it easier for third parties to understand the functions and values of services. In this case, third parties can easily understand the functions and values of each service and select appropriate APIs.

The functions supported by services need to be further simplified and clarified. That is, it is recommended that the service exposed by an API should be Interpretable-value-to- application (e.g., no/less interruption of Video watching, or higher bandwidth assurance for HD video). In this way, it helps third party to clearly know the exact benefits of SEALDD services. This makes it easier for 3rd parties to use the services.

### 4.2.2	Identified gaps

Gap 4-2-2-1: The functions supported by services should be further improved for 3rd party to understand.

## 4.3	Technical Gap#3: Direct and/or indirect interaction \with CN NFs per different deployments and requirements

### 4.3.1	Analysis

According to 3GPP TS 23.501 [10] and 3GPP TS 23.502 [11], a MNO‘s AF(trusted AF) can invoke all the APIs exposed by NF(s) from SBA interface. Thus, SEAL server(s) deployed by an MNO within the MNO’s domain may act as the trusted AF and is able to invoke network service APIs exposed by 3GPP core network functions directly over the core network SBA interface.

There is description in clause 10.2 of TS 23.434 [13] about using the NEF exposed VN group management service APIs. However, there is no any description about how to used NEF exposed VN group management service APIs in any procedures in clause 10.3.

The option of GMS as trusted AF also needs to be addressed, given network layer VN group management capabilities are needed for certain SEAL layer group service.

### 4.3.2	Gaps

Gap#4-3-2-1:	Identify the procedures from the SEAL specification to provide the direct interaction (not via NEF/SCEF) with CN NFs (e.g., PCF, SMF) and/or indirect interaction (via NEF/SCEF) per different deployments and requirements and make changes if required.

Gap#4-3-2-2: Study how to enable GMS to use 5G VN group management services when deployed within trusted domain.

Editor‘s Note: The coordination with SA2 for the supporting of trusted AF managing dynamic VN group is required if the capabilities are needed for SEAL GMS.

## 4.4	Technical gap #4: NRM service to support IoT

### 4.4.1	Analysis

1.	The network API is not designed for dedicated application scenarios(e.g., power saving) in IoT service.

Several network APIs defined in 3GPP TS 29.122 [15] can be used by the IoT service. However, they are designed in a generic manner to cater to different varieties of consumers (e.g.Vertical applications). Based on the required service, the consumers of these APIs should supply only the required input parameters and not the whole set when invoking these APIs. This makes it difficult for the application developer or the service provider to derive the required set of input parameters to provide or the service. This analysis primarily focuses on how to design the APIs catering to the need of IoT applications.

| Network API in 3GPP TS 29.122 | Description | Gap |
| --- | --- | --- |
| MonitoringEvent API | allows the SCS/AS to subscribe to notifications about specific events in 3GPP networks when the corresponding event is detected. Support multiple event (as defined in table 5.3.2.4.3-1), but only one “MonitoringType” per subscription. | - Normally, one application may have multiple events to monitor. It has to maintain multiple subscription for each event. - Events serve different purposes and no aggregation - Some events is not for IoT, NUM_OF_REGD_UES, NUM_OF_ESTD_PDU_SESSIONS, GROUP_MEMBER_LIST_CHANGE - Some events are not per UE, e.g., NUMBER_OF_UES_IN_AN_AREA |
| CpProvisioning API | allows the SCS/AS to add, change or delete the communication pattern parameter sets of the UE. | This is related with power saving for communication model with a regular interval. |
| NpConfiguration API | allows the SCS/AS to send suggested network parameters to influence certain aspects of UE/network behaviour such as the UE's PSM, extended idle mode DRX, and extended buffering configurations. | This is related power saving. |

2.	NRM is defined to simplify the utilization of network resource by 3rd party. However, there are some gaps when considering the IoT application provider as the consumer of the NRM service:

-	SS_NetworkResourceAdaptation covers all the verticals and use cases e.g., unicast, multicast/broadcast, TSC. The supporting of IoT is defined as several API operations including BDT Configuration, Unified_Traffic_Pattern_and_Monitoring_Management operation, BDT_Configuration, Device_Triggering. Considering the input parameters designed for this API it might be hard for the IoT application developer/service provider to comprehend the required parameters to invoke this API for IoT service.

-	NRM Event monitoring API has the similar issue.

-	The NRM service API granularity is not matched with the application scenarios(e.g.power saving). For a single use case, several NRM service APIs has to be invoked. For example, for power saving, the IoT appliction developer/service provider has to invoke the SS_NetworkResourceAdaptation with Unified_Traffic_Pattern_and_Monitoring_Management operation, EventMonitoring APi with power saving related input parameters.

NRM should provide IoT specific service per IoT application scenarios and requirements.

### 4.4.2	Identified gaps

-Gap#4-4-2-1: How to use the SS_NetworkResourceAdaptation and monitoringEvent API to meet IoT application requirements e.g., power savingis not clear;

## 4.5	Technical gap #5: Security Credential

### 4.5.1	Analysis

The term Security Credential was added in some SEAL service procedures and information flows, but without definition (e.g., in 3GPP TS 23.434 [3] clause 12.3.4, 3GPP TS 23.435 [4], 3GPP TS 23.436 [5], 3GPP TS 23.437 [6], 3GPP TS 23.438 [7], 3GPP TS 23.482 [13]). It’s unclear about the mechanisms on how Security Credential have been included and used for authorization and verification in application enablement services.

Table 4.5.1-1 illustrates the term Security Credential in application enablement specifications.

Table 4.5.1-1: List of Security Credential in Application Enablement Specifications

| TS | API/Functional | Clause | Summary |
| --- | --- | --- | --- |
| 23.434 | VAL server provisioning for identity management service API: SS_IdmParameterProvisioning API | 12.3.4.2, 12.3.4.3, 12.2.4.4, 12.3.4.5 | 1 API has the term "Security Credential", 15 APIs do not contain it. |
| 23.435 | Application layer network slice lifecycle management, Network slice optimization based on VAL server policy, Information collection from NSCE server(s), Predictive slice modification in edge based NSCE deployments, Multiple slices coordinated resource optimization, Predictive slice modification in Inter-PLMN based slice service continuity, Network Slice Information delivery. API: SS_NSCE_AppLayerNSLCM, SS_NSCE_Val_QoEMetrics, SS_NSCE_Val_NSLCMRecommendation, SS_NSCE_ Policy_Provisioning, SS_NSCE_NSOptimization, SS_NSCE_InfoCollection, SS_NSCE_Service_Continuity_Requirement, SS_NSCE_MultiSlices_Optimization, SS_NSCE_Inter-PLMN_Continuity API, SS_NSCE_NSInfoDelivery. | 9.4.3.2, 9.4.3.5, 9.4.3.8, 9.5.3.2, 9.5.3.11, 9.8.3.2, 9.9.3.2, 9.10.3.2, 9.13.3.2, 9.17.3.1 | 10 APIs has the term "Security Credential", 21 APIs do not contain it. |
| 23.436 | Procedures for supporting A-ADRF data storage, and edge load analytics. API: SS_AADRF_Data_Storage API SS_ ADAE_edge_analytics API | 8.8.2.2, 8.10.2.1, 8.10.2.2 | 2 APIs has the term "Security Credential", 25 APIs do not contain it. |
| 23.437 | Spatial Anchors (SAn) Server management and Spatial Map (SM) management. API: SS_SAnManagement API, SS_SAnDiscovery API, SS_SAnUsageInfo API, SS_SmManagement API, SS_SmDiscovery API, SS_SmLocalization API, SS_SmDataSourceRegistration API, SS_SmDataSourceDiscovery API, SS_SmSmasRegistration API | 8.3.1.2, 8.3.1.3, 8.3.2.2, 8.3.2.3, 8.3.3.2, 8.3.3.3, 8.3.4.2, 8.3.4.3.1, 8.3.5.2.1, 8.3.5.2.3, 8.3.5.2.4, 8.3.5.3.1, 8.3.5.3.4, 8.3.5.3.6, 8.4.2.2, 8.4.2.3.1, 8.4.3.1, 9.3.1.2, 9.3.1.3.1, 9.3.2.2, 9.3.2.3.1, 9.3.3.2, 9.3.2.3.1, 9.3.4.2, 9.3.4.3.1, 9.3.5.2, 9.3.5.3.1, 9.3.6.2.1, 9.3.6.2.3, 9.3.6.3.1, 9.3.6.3.4, 9.4.3.1, 9.5.2.2, 9.5.2.3.1, 9.5.3.2, 9.5.3.3.1, 9.5.4.2, 9.5.4.3.1, 9.5.5.2, 9.5.5.3.1, 9.5.5.3.3, 9.6.2.2, 9.6.2.3.1, 9.6.3.2, 9.6.3.3.1, 9.6.4.2, 9.6.4.3.1. | All the 9 APIs has the term "Security Credential". |
| 23.438 | Digital asset profile management, Digital asset discovery, Digital asset media management. API: SS_DAProfileManagement API SS_DADiscovery API, Digital asset media management API. | 8.2.3.1, 8.2.3.3, 8.2.3.5, 8.2.3.7, 8.3.3.1, 8.4.3.1, 8.4.3.3, 8.4.3.5, 8.4.3.7. | All the 3 APIs has the term "Security Credential". |
| 23.482 | ML model retrieval, FL member registration, FL related event subscription, and Split operation are the functions affected. API: Aimles_MLModelRetrieval API, MLR_FLMemberRegister API, MLR_FLEvents API, Aimles_SplitOperation API. | 8, 8.1, 8.2.2.2, 8.2.2.3.2, 8.2.2.3.4, 8.2.2.3.5, 8.2.3.1, 8.2.3.3, 8.2.3.6, 8.2.3.8, 8.4.2, 8.4.3, 8.4.4.2, 8.4.4.4, 8.5.5.2, 8.14.3.2, 8.14.3.4, 8.14.3.6, 8.14.3.8, 8.14.3.10, 8.14.3.12, 8.14.3.15, 8.14.3.17, 8.14.3.19, 8.14.3.21 | 4 APIs has the term "Security Credential", 22 APIs do not contain it. |

Security credential(s) for authorization and verification as well as other security mechanisms for SEAL services are specified in 3GPP TS 33.434 [13]. Moreover, stage 3 specifications for the SEAL information flows provide protocol specific security solutions, matching the defined mechanisms in 3GPP TS 33.434 [13] for the respective interfaces.

### 4.5.2	Identified gaps

Gap 4.5.2-1: How to align the Security Credential in the SEAL information flows with the security mechanisms specified in 3GPP TS 33.434 [13] and clean application enablement specifications.

## 4.6	Technical Gap#6: SEAL server API consumers

### 4.6.1	Analysis

In existing SEAL specification, the API consumer of many SEAL server APIs are restricted to VAL server only. Such restriction is not aligned with the assumption of allowing UE type of consumer, or a third party’s platform.

In addition, there are no technical support to distinguish API requests from UE side with API request from sever, nor there are requirements to support such restriction of API consumers.

### 4.6.2	Identified gaps

Gap 4.6.2-1: Solutions to guide the changes to existing SEAL specifications to cover other the SEAL server API consumers should be studied.

## 4.7	Technical gap#7: Distinguishing stage 2 SEAL services design with stage 3 API definition

### 4.7.1	Analysis

As 3GPP TS 23.502, Stage 2 API works should mainly focus on services (e.g., SEAL services) design. Services are more about what kind of resource/information are supposed to be provided to consumers and should be enabled by an API. It may also include the required input information and potiential operations for the service.While API definition is more about the API protocols and related parameters for the consumer to use services(e.g., SEAL services). API definition should belong to stage 3.

In the existing SEAL service specification 3GPP TS 23.434 [3], the SEAL services are written in the way of APIs in API clause, which mix the concept of services with APIs.  The correction to the stage 2 SEAL specifications should be made to make it aligned with 3GPP working splitting between stage 2 WGs (services design) and stage 3 WGs (API definition).

### 4.7.2	Identified gaps

Gap# 4.7.2-1: The solutions about how to modifying stage 2 SEAL specifications to align them with 3GPP working splitting between SA2 WG (services design) and stage 3 WGs (API definition).

## 4.8	Technical gap #8: API design guidelines for SEAL services

### 4.8.1	Analysis

The supporting of application ecosystem is very important for the commercialization of the SEAL technologies. So, make APIs easy-to- use is already a widely used API design principle.

There is no global unified standard/guidance about how to design easy-to-use API. However many organizations or API providers has setup their own API guidance for the easy-to use from different aspects and also announce their APIs are user friendly (e.g. https://learn.microsoft.com/en-us/azure/architecture/best-practices/api-design; https://cloud.google.com/apis/design/resources)

Defining easy-to-use should be and has already been one of key motivation of all SEAL services APIs. Clause 9 of 3GPP TS 23.222 [9] also defines several API guidelines. However, existing API design guidelines for easy-to-use needs to be further enhanced.

### 4.8.2	Identified gaps

Gap# 4-8-2-1:How to design easy-to-use APIs o guide the API definition work and provide solution about how to enhance 3GPP TS 23.222 [9].

Gap# 4-8-2-2:Revisit the existed SEAL services/APIs and make improvement based on the API design guidelines.

## 4.x	<Existing SEAL service #X>

### 4.x.1	Analysis

Editor's Note:	This clause will provide technical GAP analysis which may affect the adoption of exisisted  SEAL service X by the ecosystem partners.

Editor's Note: The Gaps can be identified from the following aspects: <<interaction with Core Network, Deployment options, adoption of APIs by other SDOs, Reason for low adoption>>

### 4.x.2	Identified gaps

Editor's Note: The Gaps identified may be also applied to other SEAL services

# 5	SEAL services adoption gap analysis

## 5.1	Generic gap #1: Representention of SEAL services within 3GPP system

### 5.1.1	Analysis

In the existing SEAL service specification 3GPP TS 23.434 [3], the architecture mainly focuses on SEAL services layer. The holistic picture providing the role and responsibility of SEAL service layer within the complete 3GPP system perspective is missing.

As the SEAL service server is a new type of 3GPP network function, the lack of holistic picture including 3GPP system may cause issues for different interpretation of SEAL services layer.

### 5.1.2	Identified gaps

Gap# 5-1-2-1:The representation of SEAL services layer within 3GPP system needs to be added.

## 5.2	Void

## 5.3	Adoption gap #2: Representation of SEAL services within the whole 3GPP network exposure system

### 5.3.1	Analysis

The important role of SEAL services is to re-expose the 3GPP system capabilities by combining multiple capabilities of 3GPP system, across different domain including core network, OAM and terminal client.

In the existed SEAL service specification, the architecture mainly focuses on SEAL services layer. The information to tell the role and responsibility of SEAL service layer from the whole 3GPP system perspective are missing.

Using a layered representation of whole 3GPP network exposure system in the network exposure language will be helpful for customers to understand SEAL services better.

### 5.3.2	Identified gaps

Gap# 5-3-2-1:How to represent the SEAL service layer in the whole 3GPP network exposure system needs to be added

## 5.4	Adoption gap #3: Linking SEAL services to target application use cases

### 5.4.1	Analysis

Many third-party application providers lack a comprehensive understanding of the mechanisms provided by 3GPP. A more detailed knowledge of SEAL could be highly beneficial for application layer developers, enabling them to make more effective use of its capabilities. Additionally, identifying and analyzing relevant use cases could lead to significant improvements in SEAL functionality. As an example; for location services, developers are often unaware of the distinctions and benefits with usage of SEAL compared to 3GPP Core Network location services or alternative OTT location services.

### 5.4.2	Identified gaps

Gap#5-4-2-1: It is essential to study how SEAL services benefit application developers (e.g., streamlined development time, enhanced robustness and reliability, abstraction of complicated APIs, etc.) and explore what new use cases can be enabled by these advantages.

Editor's Note: Solutions for this key issue should be per SEAL services.

## 5.5	Adoption Gap#5: Incomplete business relationship for SEAL service

### 5.5.1	Analysis

In 3GPP TS 23.434 [3] clause 5 describes the business relationship between different stakeholders. In clause 5 the SEAL service provider is quite not clear and complete (e.g., the role of seal service provider, seal clients/server provider). For example, the existing business relationship in the specification does not involve all the stakeholder e.g., the SEAL client providers.

The SEAL client may be potentially provided by different software providers, e.g., OS, 3rd party middleware provider, application provider, or substituted by other alternatives to serve different business relationship.

### 5.5.2	Identified gaps

Gap#5-5-2-1: Identify the further enhancements (like role of seal client provider) for the seal service provider described in clause 5 based on the different seal service deployment options in the context of the overall SEAL business relationship.

## 5.6	Adoption Gap#6: Deployment options involving SEAL Client

### 5.6.1	Analysis

Existing deployment defined in clause 8 of 3GPP TS 23.434 [3] does not cover the SEAL Client.

### 5.6.2	Identified gaps

Gap#5-6-2-1: Study how to cover SEAL client in the deployment mode.

## 5.7	Generic gap#7: The adoption of SEAL layer by External Organizations

### 5.7.1	Analysis

Both SEAL and the Network Transformation functions of the GSMA Open Gateway Platform are defined to provide abstraction layer services to enable third parties to use 5G/5GA capabilities. The current GSMA Open Gateway Platform Network Transformation functions are providing the mapping between CAMARA APIs (adopted by GSMA Open Gateway as NBIs) and NEF/SCEF APIs (adopted as SBI).

The SEAL services have not been adopted by GSMA Open Gateway. Assuming that GSMA Open Gateway might be interested to use SEAL APIs in the future, SA6 should analyse how SEAL could support the GSMA Open Gateway Platform.

Besides GSMA, the organizations like 5GACIA/5GAA are also potentical consumers of SEAL. How to map the SEAL layer to their ecosystems should also be made.

### 5.7.2	Identified gaps

Gap#5-7-2-1: Study the different alternatives of using the SEAL layer in external organizations(e.g., GSMA Open Gateway Platform,5GAA/5GACIA etc), using the GSMA OPG architecture as base, and potential advantages/dis-advantages of each alternative.

# 6	Void

# 7	Architecture requirements

Editor's Note: requirements can be based on the idetified gaps

## 7.1	< API design guidance> requirements

Editor's Note:	This clause will describe the architectural requirements for the studied application layer capabilities.

[Requirement 5.X.1] All the SEAL services and APIs should be defined in the principle of simple, consistent and easy-to-use.

# 8	Solutions

<<for the idetified gaps and architecture requirements>>

## 8.0	Solutions and gaps mapping

This clause provides the mapping of solutions to key issues.

Table 8.0-1: Mapping of solutions to technical gaps

|  | Gap#1 | Gap#2 | Gap#3 | Gap#4 | Gap#5 | Gap#6 | Gap#7 | Gap#8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Sol #1 |  | X |  |  |  |  |  |  |
| Technical Sol #2 |  |  |  |  |  |  |  | X |
| Technical Sol #3 |  |  |  |  |  |  |  | X |
| Technical Sol #4 |  |  | X |  |  |  |  |  |
| Technical Sol #5 |  |  |  |  | X |  |  |  |
| Technical Sol #6 | X |  |  |  |  |  |  |  |
| Technical Sol #7 | X |  |  |  |  |  |  |  |
| Technical Sol #8 |  |  |  |  |  | X |  |  |
| Technical Sol #9 |  |  |  | X |  |  |  |  |
| Technical Sol #10 | X |  |  |  |  |  |  |  |
| Technical Sol #11 |  |  |  | X |  |  |  |  |
| Technical Sol #12 |  |  |  |  |  |  | X |  |
| Technical Sol #13 |  |  |  |  |  |  |  | X |

Table 8.0-2: Mapping of solutions to adoption gaps

|  | Gap#1 | Gap#2 | Gap#3 | Gap#4 | Gap#5 | Gap#6 | Gap#7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Adoption Sol #1 | X |  |  |  |  |  |  |
| Adoption Sol #2 |  | X |  |  |  |  |  |
| Adoption Sol #3 |  |  |  |  | X |  |  |
| Adoption Sol #4 |  |  |  |  |  | X |  |
| Adoption Sol #5 |  |  |  | X |  |  |  |
| Adoption Sol #6 |  |  |  |  |  |  | X |

## 8.1	Solutions for SEAL services technical gaps

### 8.1.1	Technical solution#1 SEALDD APIs improvement

#### 8.1.1.1	Solution description

This solution is corresponding to Gap#4.2.2-1.

.

According to SA1 identified 5G applications/Use cases, the application needs from different application includes the following:

a) Application layer signalling between AC and AS for different applications with different data transmission QoS requirements:

a.1) Best effort demanding applications

a.2) Low latency demanding applications like signalling for MCX call

- Transmitting the application layer data for different applications with different data transmission QoS requirements:

b.1) Low latency and high reliable demanding application like remote robot/machine control.

b.2) High data rate with low latency demanding application like Realtime XR video applications, Machine vision application….

b.3) Low latency like video call/voice call.

b.4) Low cost demanding like background data

b.5) Low power demanding like IoT device applications.

b.6) Multiple flows synchronisation transmission service like multi-modal XR applications

Currently some of the SEALDD services are already defined based on the application needs as the table 8.1.1.1-1.

Table 8.1.1.1-1: List of Rel-18 and Rel-19 services in SEAL-DD

| Required services by Application requirements | Service available | NOTES |
| --- | --- | --- |
| a.1 | Not yet | —— |
| a.2 | Sdd_RegularTransmission in 3GPP TS 23.433 | APIs name may cause misleading |
| b.1 | Sdd_URLLCXX in 3GPP TS 23.433 | —— |
| b.2 | Not yet | —— |
| b.3 | Not yet | —— |
| b.4 | Sdd_BDT_X in 3GPP TS 23.433 | —— |
| b.5 | Not yet | SEALDD capability for Low power is not supported yet and should be added within Rel20 |
| b.6 | Sdd_multi-modalTransmission | —— |

The following new SEALDD services should be added.

#### 8.1.2.1 SEALDD regular connection management.

##### 8.1.2.1.1	General

The following clauses specify procedures, information flow and APIs for establishing an SEALDD enabled end-to-end connection between VAL client and VAL server. The end-to-end connection is uniquely identified in the SEALDD layer by the SEALDD-UU flow ID. The specific procedures detailed in the subsequent clauses are for cases in which the SEALDD regular connection is used respectively for application signalling, application data delivery initiated by VAL server, and application data delivery initiated based on DD policy.

Many applications have no performance requirement for the application signalling interactions, so the best effort signalling transmission is sufficient. There are also some applications like mission critical application which desire low latency and reliable performance on the signalling interactions, so the reliable signalling transmission is required.

In order to meeting the two kinds of signalling interactions requirements, the SEALDD provides two signalling deliver services i.e., best effort signalling delivery and reliable signalling delivery accordingly.

As for the signalling transmission connection establishment, the SEALDD enabler layer can determine to establish different signalling transmission connection for the VAL server based on the signalling transmission requirement.

##### 8.1.1.2.2	Procedure

###### 8.1.1.2.2.1	SEALDD enabled best effort signalling transmission connection establishment procedure

Figure 8.1.1.2.2.1-1 illustrates the procedure for best effort signalling transmission connection establishment. In this clause, considering VAL server’s requirement on signalling transmission is not strict, then the VAL server can invoke best effort signalling transmission connection establishment API to enable the connection establishment.

Pre-condition:

-	The VAL server can discover and select the SEALDD server by CAPIF functions.

Figure 8.1.1.1.2.1-1: SEALDD best effort signalling transmission connection establishment procedure

1.	The VAL server decides to use SEALDD service for application signalling transfer and allocates address as SEALDD-S connection information for receiving the application signalling packets from SEALDD server. The VAL server sends SEALDD enabled best effort signalling transmission connection establishment request to the SEALDD server. The SEALDD enabled best effort signalling transmission connection establishment request includes the VAL server ID, VAL service ID to identify the VAL application traffic, the SEALDD-S Data transmission connection information of the VAL server side.

2.	Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, the SEALDD server allocates a specific address used for SEALDD traffic transfer with the incoming SEALDD client(s) for the VAL server and responds with a SEALDD enabled best effort signalling transmission connection establishment response.

3-4.	Same as step 3-4 of clause 9.2.2.1 in 3GPP TS 23.433 [2].

5.	The SEALDD client allocates SEALDD-UU flow ID(s) mapping to application signalling traffic for application signalling transmission. The SEALDD client sends SEALDD signalling transmission connection establishment initiated by SEALDD client request to SEALDD server with the SEALDD client ID, the SEALDD-UU flow ID(s), VAL server ID, VAL service ID and the SEALDD traffic descriptor of the SEALDD client side (the SEALDD-S connection information of the SEALDD client for receiving the downlink SEALDD-UU signalling traffic). The request message also contains the selected VAL server endpoint information. The SEALDD server retrieves the location information of the VAL UE or SEALDD client from SEAL LM services defined in 3GPP TS 23.434 clause 9.3.12 and verifies with the Geofence policy configured by the VAL server to allow or restrict the signalling connection establishment. If the location information is allowed as per the configured geofence policy then the SEALDD server allows the signalling connection establishment, otherwise SEALDD server returns failed result e.g. performs connection reject.

6-8.	Same as step 6-8 of clause 9.2.2.1 in 3GPP TS 23.433 [2].

##### 8.1.1.1.3	Signalling flow

###### 8.1.1.1.3.1	SEALDD enabled best effort signalling transmission connection establishment request

Table 8.3.1.1.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting the best effort application signalling traffic transmission service.

Table 8.3.1.1.3.1-1: SEALDD enabled best effort signalling transmission connection establishment request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service |
| Identity | O | Identifier of specific UE or VAL user |
| SEALDD-S connection information | M | Address information (e.g., IP address and/or port, URL) of the VAL server to receive the signalling traffic from the SEALDD server |
| Requested 5GS Delay | O | The Requested 5GS Delay provided by the VAL server |

###### 8.1.1.1.3.2	SEALDD enabled best effort signalling transmission connection establishment response

Table 8.3.1.1.3.2-1 describes the information flow from the SEALDD server to the VAL server for responding to the best effort application signalling traffic transmission.

Table 8.1.1.1.3.2-1: SEALDD enabled best effort signalling transmission connection establishment response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| SEALDD-S connection information | O (See NOTE1) | Address information (e.g., IP address and/or port, URL) of the SEALDD server to receive the signalling traffic from the VAL server |
| Cause | O (See NOTE2) | Indicates the reason for the failure, e.g., SEALDD policy mismatch. |
| NOTE1: Information element shall be present when the Result is successful. NOTE2: Information element shall be present when the Result is failure. |  |  |

###### 8.1.1.1.3.3	SEALDD signalling transmission connection establishment initiated by SEALDD client request

Table 8.3.1.1.3.3-1 describes the information flow from the SEALDD client to the SEALDD server for requesting the SEALDD signalling transmission connection establishment.

Table 8.1.1.1.3.3-1: SEALDD signalling transmission connection establishment initiated by SEALDD client request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identity of the requestor (SEALDD client or SEALDD server). |
| SEALDD-UU flow ID | M (See NOTE 1) | Identity of the SEALDD-UU flow. |
| VAL server ID | O | Identity of the VAL server, applicable for SEALDD client-side initiated request. |
| VAL service ID | O | Identity of the VAL service |
| Selected VAL server endpoint | M | Endpoint of the selected VAL server |
| SEALDD traffic descriptor | O | SEALDD traffic descriptor (e.g., address, port, URL, transport layer protocol) of the SEALDD client side (for client-side initiated request) |
| Identity | O | The VAL user ID of the VAL user or VAL UE ID. |
| SEALDD communication lifetime | O | Identifies the DD communication lifetime, applicable for SEALDD server-side initiated request. |
| NOTE 1: The SEALDD-UU flow ID is used by the SEALDD client and SEALDD server to identify different application traffic, and it is mapped to the identifiers of the application traffic and data transmission session. |  |  |

###### 8.1.1.1.3.4	SEALDD signalling transmission connection establishment initiated by SEALDD client response

Table 8.3.1.1.3.4-1 describes the information flow from the SEALDD server to the SEALDD client for responding to the SEALDD signalling transmission connection establishment initiated by SEALDD client.

Table 8.1.1.1.3.4-1: SEALDD signalling transmission connection establishment initiated by SEALDD client response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of establishing the SEALDD connection. |
| SEALDD traffic descriptor | O (See NOTE1) | SEALDD traffic descriptor (e.g., address, port, URL, transport layer protocol) of the SEALDD server side |
| Pending timer | O (See NOTE 2) | The pending timer to trigger the re-connection from SEALDD client when bandwidth limit check is failed. |
| Cause | O (See NOTE 3) | Indicates the reason for the failure |
| NOTE 1 Information element shall be present when the Result is successful. NOTE 2: The Pending Timer is used for the SEALDD enabled bandwidth control for different VAL users, applicable for client-side initiated request. NOTE 3: Information element shall be present when the Result is failure. |  |  |

###### 8.1.1.1.2.2	SEALDD enabled High data rate & low latency (HRLC)

The SEALDD enabled regular data transmission defined in clause 9.2.2.2	of 3GPP TS 23.433[2] can be used with the following changes:

-	The term Sdd_RegularTransmission is changed to Sdd_HRLCTransmission.

-	The QoS information for the application traffic in step 1 of 9.2.3.1 and 9.3.3.3 should restricted to indicate required data rate & latency.

###### 8.1.1.1.2.3	SEALDD enabled application data transmission for Low latency (LC)

The SEALDD enabled regular data transmission defined in clause 9.2.2.2	of 3GPP TS 23.433[2] can be used with the following changes:

-	The term Sdd_RegularTransmission is changed to Sdd_LCTransmission.

-	The QoS information for the application traffic in step 1 of 9.2.3.1 and 9.3.3.3 should restricted to indicate required latency.

#### 8.1.1.2	Solution evaluation

This solution provides new SEALDD services, which are designed according to application needs on performance. Different SEALDD services also requires different resource cost.

#### Therefore, it is easily for SEALDD services consumer to understand what benefits their applications can get, and why they are charged differently.

### 8.1.2	Technical solution #2: < SEAL services and SEAL API design guidelines>

#### 8.1.2.1	Solution description

This solution addresses the technical gaps#8in clause 4.8. This solution is about how to establishing common evaluation criteria to support easy-to-use SEAL service and associated APIs.

The difference and relationship between SEAL service design (about what information/resource can be exposed to third party) and API definition for implementation (about how to enable the consumer to make request for the service) is as below:

- SEAL service mainly focuses on the capabilities and how to use the capabilities to provide service to fulfil serve different application requirements (e.g., Stage 2 may define location tracing service for a single user. Stage 2 can also decide to further support group location service based on group management capability and location capability). It is more about what kind of information/resource that applications can receive.

- While API definition mainly focuses on how to define the API associated with a SEAL service. It is more about the method how the third party can request the required information/resource for a SEAL service from provider. It is up to stage 3 to define the APIs to enable the third party to use the stage 2 designed service.

APP developer is one of the important customers of SEAL services. The s the application implementation benefits a lot from service complying with the following guidelines:

-	Each of the SEAL services offered by a SEAL Function shall be self-contained (e.g., contain required parameters to support the service, extensible and evolvable), reusable and use management schemes independently of other SEAL services offered by the same SEAL Function.

-	The expected impact to implementation of Application shall be minimized.

-  Simplicity For Consumers: Keeping the services design simple and intuitive with clear naming conventions and logical organization of functionalities contribute to simplicity.

-	Define the essential input information for Service Functionality: Identify and specify only the input information necessary to use the service.

-	SEAL services provide value added services based on 5G/5GA network capabilities (including 5G core and OAM. The SEAL services can be designed in the following way:

-	Application layer needs oriented

The application layer requirements on 5G/5GA capabilities include data transmission performance optimization related, or information/resource related. An application layer needs-oriented API service can be designed to provide certain optimization on the performance of the transmission of application data (e.g., Reducing latency, or improving reliability), or providing the information (e.g., network statues) which is required by application(s).

NOTE:	Application layer needs-oriented service(s) should be proposed with the justification of the use case(s) for the requirements.

The APIs defined according to the following API guidelines should be complied:

- Comply with (or adapts to) the API paradigm used by application.

- Minimize unnecessary attributes to simplify the use and avoid error of testing.

#### 8.1.2.2	Solution evaluation

The guidelines defined in this solution are aligned with the principle of defining easy-to-use service(s).

NOTE:	The API service design guidelines will only impact the newly added services and will not impact the existing architecture or API services.

### 8.1.3	Technical solution#3: SEAL service design guidance framework

#### 8.1.3.1	Description

##### 8.1.3.1.1	Aspects related to Guidance Framework

###### 8.1.3.1.1.1	General

This guidance framework addresses technical gap#8 by providing a systematic approach to help application developers understand and utilize SEAL services effectively. The guidance explains service applicability through detailed scenario analysis, presents comparative advantages over existing solutions, and offers practical implementation recommendations.

###### 8.1.3.1.1.2	Service Applicability Analysis

The first aspect of this guidance is to enumerate potential application scenarios and industries for each SEAL service, enabling developers from relevant domains to quickly understand the applicability of these services.

For example, when describing the location management service, we identify its applicability in scenarios such as indoor positioning, fleet management, and emergency response. By providing such comprehensive scenario mapping, developers can easily identify which SEAL services align with their specific use cases and industry requirements.

###### 8.1.3.1.1.3	Comparative Advantages

The second aspect of this guidance is to describe the advantages of SEAL services by comparing them with similar solutions (such as 3GPP core network services and OTT services).

When compared with 3GPP core network services, SEAL services demonstrate advantages in API simplification, enhanced reliability, and unified authentication. In contrast to OTT services, SEAL services provide superior QoS guarantees and deeper network integration. These advantages are illustrated through concrete examples and performance metrics in different application contexts.

Through explicit comparative analysis, developers can more quickly identify and adopt SEAL services that best suit their needs.

###### 8.1.3.1.1.4	Development Guidance

The third aspect of this guidance is to provide best practice examples for some specific scenarios.

For each SEAL service, we select specific scenarios and provide best practice examples to ensure efficient integration and optimal usage. These guidelines include detailed workflows that outline API interactions step by step, ensuring consistency and reliability. Additionally, we offer architecture-level deployment recommendations, covering scalability, security, and performance considerations tailored to different environments.

By following these best practices, developers can quickly understand SEAL services, accelerate implementation, and seamlessly integrate them into existing systems.

###### 8.1.3.1.2	Implementation of guidance framework

Editor's Note: This clause is FFS

#### 8.1.3.2	Solution evaluation

This solution addresses technical gap#8by proposing a guidance framework for SEAL service design, which helps application developers understand and utilize SEAL services effectively.

### 8.1.4	Technical solution 4: The specification changes for direct interaction between LMS and GMLC/LMF

#### 8.1.4.1	Solution description

This solution is to address LMS related content for Gap#4-3-2-1. The solution is descibed in the way to tell how to change the 3GPP TS 23.434 [3] with highlight content.

##### 8.1.4.1.1	Changes to cluse 9.2.2 of 3GPP TS 23.434 [3]

The changes to Clause 9.2.5 of 3GPP TS 23.434 [3] should be changed as the following (new text showed in the bold text).

*******************************Begin of Changes***************************************************

9.2.2	On-network functional model description

Figure 9.2.2-1 illustrates the generic on-network functional model for location management.

Editor's Note 1: The above figure should be deleted by the normative CR to 3GPP TS 23.434 [3].

Figure 9.2.2-1: On-network functional model for location management

The location management client communicates with the location management server over the LM-UU reference point. The location management client provides the support for location management functions to the VAL client(s) over LM-C reference point. The VAL server(s) communicate with the location management server over the LM-S reference point. The VAL client communicates with the VAL server over the VAL-UU reference point which is outside the scope of this document.

The location management server communicates with the SCEF via T8 reference point to obtain location information from the underlying 3GPP network system. The location management server obtains location information from the NEF via N33 reference point by mechanism defined in clause 5.2.6.2 of 3GPP TS 23.502 [11]. The location management server may obtain location information from the GMLC via Le reference point defined in clause 4.4.1 of 3GPP TS 23.273 [50]. When the Location management server is deployed within operator’s trusted domain, it may  obtain location information from the GMLC over Ngmlc or LMF over Nlmf.. The location management server may optionally obtains location information from the 3rd party location server via LM-3P reference point.

When the fused location function is present, the location management server, may use the location information from multiple sources to determine a more accurate UE location. The fused location function may select one or more location sources and location methods based on the requested location QoS which obtained from the location management server.

The location management server supports to report the requested value-added location information (e.g., Geofencing monitoring events, history location data, prediction related to UE location, etc.) to the VAL server over LM-S reference point.

NOTE 1:	Location information from LCS of 4G system is not exposed by SCEF.

NOTE 2:	Non-3GPP access used by the UE is out of scope of the present document.

******************************End of Changes***************************************************

##### 8.1.4.1.2	Changes to cluse 9.2.5 of 3GPP TS 23.434 [3]

The changes to Clause 9.2.5 of 3GPP TS 23.434 [3] should be changed as the following (new text showed in the bold text).

******************************Begin of Changes***************************************************

9.2.5	Reference points description

9.2.5.1	General

The reference points for the functional model for location management are described in the following subclauses.

9.2.5.2	LM-UU

The interactions related to location management functions between the location management client and the location management server are supported by LM-UU reference point. This reference point utilizes Uu reference point as described in 3GPP TS 23.401 [9] and 3GPP TS 23.501 [10].

LM-UU reference point provides a means for the location management server to receive location information report from the location management client. The LM-UU reference point shall use SIP-1 and SIP-2 reference points for subscription/notification related signalling. And for transport and routing of location management related signalling LM-UU reference point uses the HTTP-1 and HTTP-2 signalling control plane reference points.

9.2.5.3	LM-PC5

The interactions related to location management functions between the location management clients located in different VAL UEs are supported by LM-PC5 reference point. This reference point utilizes PC5 reference point as described in 3GPP TS 23.303 [12].

9.2.5.4	LM-C

The interactions related to location management functions between the VAL client(s) and the location management client within a VAL UE are supported by LM-C reference point.

9.2.5.5	LM-S

The interactions related to location management functions between the VAL server(s) and the location management server are supported by LM-S reference point. This reference point is an instance of CAPIF-2 reference point as specified in 3GPP TS 23.222 [8].

LM-S reference point is used by the VAL server to request and receive location information from location management server. The LM-S reference point shall use SIP-1 and SIP-2 reference points for subscription/notification related signalling. And for transport and routing of location management related signalling LM-S reference point uses the HTTP-1 and HTTP-2 signalling control plane reference points.

9.2.5.6	LM-E

The interactions related to location management functions between the location management servers in a distributed deployment are supported by LM-E reference point.

9.2.5.7	T8

The reference point T8 supports the interactions between the location management server and the SCEF and is specified in 3GPP TS 23.682 [13]. The functions related to location management of T8 are supported by the location management server.

9.2.5.8	Le

The reference point Le supports the interactions between the location management server and the GMLC and is specified in 3GPP TS 23.273 [50].

9.2.5.9	LM-3P

The reference point LM-3P is used for location retrieval of the target UE from a third-party location server.

NOTE:	Further definition of LM-3P is out of scope of this specification.

9.2.5.10	N33

The reference point N33 supports the interactions between the location management server and the NEF and is specified in 3GPP TS 23.501 [10] and 3GPP TS 23.502 [11]. The functions related to location management of N33 are supported by the location management server.

9.2.5.11	Ngmlc

The reference point N33 supports the interactions between the location management server and the GMLC and is specified in 3GPP TS 23.273 [8].

******************************End of Changes***************************************************

##### 8.1.4.1.3	Changes to cluse 9.3.7 of 3GPP TS 23.434 [3]

The changes to Clause 9.3.7 of 3GPP TS 23.434 [3] should be changed as the following (new text showed in bold text).

******************************Begin of Changes***************************************************

9.3.7	Location information subscription procedure

Figure 9.3.7-1 illustrates the high-level procedure of location information subscription request. The same procedure can be applied for location management client and other entities that would like to subscribe to VAL user or VAL UE location information. This procedure is also used for initiating tracking a UE's location. The procedure considers target UE location information provided by its surrounding UEs, the procedure can improve location accuracy and can be used also for target UE temporarily out of 3GPP radio coverage. Location management client 1 is used as example in the figure as target UE that the VAL server is interested to know its location information.

Figure 9.3.7-1: Location information subscription request procedure

1.	The VAL server sends a location information subscription request to the location management server to subscribe location information of one or more VAL users or VAL UEs. The request may include an indication for supplementary location information, the location QoS which contains the location accuracy, response Time and QoS class as defined in clause 4.1b of TS 23.273 [50], the velocity information of target UEs, and an indication whether the statistic of target UE location is needed per e.g., temporal/spatial granularity. If the indication whether the statistic of target UE location is needed, the corresponding time/location information should also be included. In addition, the VAL server may request velocity information of the VAL UEs/Users in step 1.

2.	The location management server shall check if the VAL server is authorized to initiate the location information subscription request and may retrieve the available and proper location access type and positioning methods (e.g., as described in TS 23.273 [50] and TS 29.572 [51]) for the target VAL UE based on the received location QoS. Further, the location management server may initiate location reporting configuration optionally including the retrieved location access type and positioning methods with the location management client of the UE for immediate reporting.

3.	The location management server may optionally subscribe for UE location information from 3GPP core network for the UE. If the indication for supplementary location information is included in step 1, then UE location information is obtained from the 3GPP core network and/or the 3rd party location management server.

If the velocity information of target UE is requested in step 1, the location management server asks the location management client or 3GPP core network to report the target UE location data with the requested velocity information.

NOTE 1:	How the location management server obtains the UE location from the 3rd party location management server is out of scope of this specification.

4.	The location management server determines the UE location information of the UE as received in steps 2 and 3 and checks if it meets the location QoS requirements (if any) or not.

If the indication whether the statistic of target UE location is included in step 1, the location management server performs the following actions based on different granularity.

If the indication whether the statistic of target UE location is needed per temporal granularity is included in step 1, the location management server calculates the UE location data received in step 3 per the temporal granularity with the requested time condition.

If the indication whether the statistic of target UE location is needed per spatial granularity is included in step 1, the location management server calculates the UE location data received in step 3 per the spatial granularity with the requested location information.

5.	The location management server may optionally request the UE location from its surrounding UEs (as described in step 5 to 9). The location management server decides how to retrieve surrounding UEs which may be based on location management client 1 registered positioning method in location service registration. If the location management server decides to use UE-assisted surrounding UE retrieval, step 6 is executed; otherwise, NW-assisted surrounding UE retrieval is used as described in step 7.

6.	The location management server requests the location management client 1 with an optional surrounding UE retrieval method (e.g., Prose, BT, WIFI) to provide its surrounding UEs and the location management client 1 responds the location management server with VAL UE/User ID of discovered UEs as described in clause 9.3.x;

NOTE 2:	For Prose capable UE, the User Info ID defined in 3GPP TS 23.586 [57] can represent VAL user ID. For Bluetooth capable UE, the Bluetooth MAC address can represent VAL UE ID. For Wi-Fi33 Direct capable UE, the WiFi MAC address can represent VAL UE ID.

7a.	The location management server derives an appropriate location area (where UE to UE communication is possible) internally using the location information of UE 1 (determined in step 4) as reference location. If the UE 1 location is last known UE location, the location management server uses UE mobility analytics service from 3GPP CN to derive UE location estimation for UE 1.

7b.	The location management server uses 3GPP CN service (NEF service in untrusted mode or GMLC service in trusted mode of Number of UEs present in a geographical area) to obtain all UEs within the derived location area in step 6.

8.	The location management server selects a set of UEs (location management client 2...n) from UEs obtained in step 5 or step 7 based on registered location service information (e.g., positioning method) from location management client 2...n. Then location management server sends location request to location management client 2...n using On-demand location reporting procedure (as defined in clause 9.3.4) to obtain UE 1 location including an optional UE positioning method (e.g., PC5 SEAL LM, BT, WIFI) to be used to retrieve location.

NOTE 3:	The location management server can, based on local policy, select a UE without LM registration and trigger location request to the UE. If so, the request can be ignored by the UE (e.g., if SEAL LM-UU service is not supported) or the UE can reject the request (if the required positioning method is not supported).

9.	The location management server askes the location information of UE 1 reported by location management client 2...n into consideration to calibrate UE 1 location determined in step 4.

NOTE 4:	It is assumed that different location information collected over different positioning method may have differences in accuracy. How location management server processes calibration is implementation specific.

10.	The location management server replies with a location information subscription response indicating the subscription status and if immediate reporting was requested, the location information of the VAL UE(s).

NOTE 5:	The VAL server can use obtained UE location and velocity in application specific ways (e.g., traffic monitoring in V2X).

******************************End of Changes***************************************************

##### 8.1.4.1.4	Changes to cluse 9.3.11 of 3GPP TS 23.434 [3]

The changes to Clause 9.3.11.2 of 3GPP TS 23.434 [3] should be changed as the following (changes showed in bold text).

************************************begin of changes*********************************************

9.3.11.2	Monitoring Location Deviation procedure

Figure 9.3.11.2-1 describes the procedure for monitoring the VAL UE's location in a given area of interest.

Pre-condition:

-	The location management server authorized to consume the 3GPP core network service (Monitoring events provided by NEF as specified in 3GPP TS 23.502 [11] or by GMLC as specified in 3GPP TS 23.273).

Figure 9.3.11.2-1: Monitoring VAL UE's location at a given location

1.	The VAL server sends Monitor Location Subscription Request to location management server including target VAL UE Identifier(s), predetermined or dynamic area of interest information, notification interval and notification URI where the VAL server intends to receive the notifications from LM server regarding target VAL UE's presence in a given area, interested notification information, dynamic geofencing conditions.

-	"Area of interest" is the location information, which the VAL server wishes to monitor the target VAL UE's location adherence. This parameter can include an area of interest information and other relevant parameters. In the case of dynamic area of interest, it includes a reference VAL UE Identifier and its proximity range.

-	"Notify_Interval" represents the periodic interval in which the location management server needs to notify target VAL UE's location information to the VAL server. When the target VAL UE moves away from the "Area of interest", then the location management server ignores the "Notify_Interval" and sends the location notification to the VAL server immediately.

-	"dynamic geofencing conditions" includes a temporal condition(s) (e.g., time of day and day of week), connectivity condition(s) (e.g., operator information, roaming status, access type) and/or a spatial condition(s) (e.g., location area(s)) for the reference VAL UE. For instance, the monitoring is valid when a reference VAL UE is served by operator A without using satellite in location area X from 7:00am to 9:00am on working days.

2.	If dynamic area of interest information and dynamic geofencing conditions are received, the location management server requests 3GPP CN (e.g., NEF/GMLC service for location, access type and roaming status monitoring as specified in 3GPP TS 23.502 [11]) to monitor the reference VAL UE location change, access type change and roaming status change, in order to determine spatial dynamicity in area of interest related to the target UE(s). If spatial condition is received, the location management server also requests 3GPP CN (i.e., NEF/GMLC service for area of interest monitoring as specified in 3GPP TS 23.502 [11]) to know whether the reference VAL UE is in the VAL server's predetermined area (e.g., PLMN x). The location management server will process the monitoring considering the received dynamic geofencing conditions for the reference UE.

3.	If dynamic area of interest information is received, the location management server requests 3GPP CN (i.e., NEF/GMLC service for loss of connectivity as specified in 3GPP TS 23.502 [11]) for both target UE(s) and reference UE.

4a.	Location management server processes the Area of interest information in the request, and then subscribes to target VAL UE location monitoring as specified in 3GPP TS 23.502 [11] with appropriate parameters mapping. Based on the subscription, the location management server receives the target VAL UE location information periodically from the 3GPP core network. The LM server also invokes NEF/GMLC service for Area of Interest monitoring as specified in clause 5.3.3 of 3GPP TS 23.256 [52] to know whether the target VAL UE is in the VAL server's predetermined or dynamic area of interest.

4b.	LM server shall use the Location information procedures as specified in clause 9.3.7 and clause 9.3.10, to periodically obtain the target VAL UE location information. Based on the geographic information from the VAL server, the LM server may determine to additionally include the positioning methods in SEAL LMS procedures to obtain location information.

4c.	LMS may periodically obtain the target VAL UE location information from the 3rd party location management server based on the geographic information from the VAL server.

4d.	LM server may configure the off-network position and report policy to the target VAL UE and/or reference VAL UE as described in clause 9.3.21.3.1 when they still have connectivity. This is to initiate the UE-only Operation SL/Ranging as described in 3GPP TS 23.586 [57], cache the location, report the location result when the connectivity is back. Time duration is included if the Dynamic geofencing conditions is specified in step 1.

NOTE 1:	How the LMS obtains the target VAL UE location from the 3rd party location management server is out of scope of this specification.

NOTE 2:	Step 4a to 4c are performed for each target VAL UE if multiple target VAL UE Identifiers are provided by the VAL server.

5.	Location management server, after successful subscription according to steps 2~4, sends Monitor Location Subscription response, indicating that the location management server accepts VAL server's request and will monitor the location of the target VAL UE(s) to verify if the target VAL UE(s) is in the area of interest.

NOTE 3: Either step 6 or step 7 is performed if it is notified about loss of connectivity for the reference VAL UE and target VAL UE(s).

6.	The location management server requests the target VAL UE to obtain the reference UE's location information (i.e., the absolute location or the relative location).

NOTE 4:	In this figure, it assumes the reference VAL UE lost connectivity and the target VAL UE still has the connectivity. If the target VAL UE lost connectivity and the reference VAL UE has the connectivity, location management server requests the reference VAL UE to obtain the target VAL UE's location information.

6a.	The location management server determines the reference VAL UE lost connectivity and the target VAL UE still has the connectivity, the location management server sends location subscription request to the target VAL UE. The reference VAL UE identity and the Ranging/SL positioning method, location type (i.e., the absolute location or the relative location) are included. The location management client in the target VAL UE returns a location subscription response.

6b.	The target VAL UE initiates the Ranging/SL based MO-LR procedure as described in clause 6.20 of 3GPP TS23.273 [50] to obtain the reference VAL UE location information. The target VAL UE obtains the reference UE's location information.

6c.	The target VAL UE sends the location notification with the reference UE's location information.

7.	Alternatively, the LM server requests the target UE's location information from the core network. The core network initiates the Ranging/SL based MT-LR procedure as described in clause 6.20 of 3GPP TS 23.273 [50] and obtains the target VAL UE location information.

8.	Location management server processes the location information received from SEAL Location Information procedures, the 3GPP core network and/or the 3rd party location management server, and validates the information. If the location information is matching, then the Location management server shall check if the VAL UE's current location is within the predetermined or dynamic area of interest received in step 1.

NOTE 4:	The location information received from the 3GPP core network and from location management client does not necessarily be exactly the same, the location management server can aggregate the location information if there is no big deviation.

Location management server verifies its determined target VAL UE location status (i.e., presence or absence with regard to the predetermined area of interest) with the retrieved 3GPP core network result from the NEF/GMLC monitoring service for Area of Interest. The location management server will continue with step 7, step 8 and step 9 as applicable.

9a.	If the location information received from location management client, the 3GPP core network and/or the 3rd party location management server do not match or the NEF/GMLC reported Area of Interest monitoring result does not match with the determined target VAL UE location status (present/absence) in location management server, then the location management server shall consider the target VAL UE as outside from its specified area of interest and shall notify ("Notify Mismatch Location" message) the VAL server of the same, including target VAL UE ID and the location information from location management server, the 3GPP core network and/or the 3rd party location management server in the notification message.

9b.	If the target VAL UE's current location is from location management client, the 3GPP core network and/or the 3rd party location management server matches, and either not in the predetermined area of interest received from VAL server in Monitor Location Subscription Request message or not within the proximity range of the reference UE, and such out of area of interest result is also aligned with the NEF/GMLC reported area of interest monitoring result, then the location management server considers the target VAL UE as outside from its specified area of interest and shall notify the VAL server that the target VAL UE's current location is outside of area of interest and target VAL UE ID in "Notify Absence" message.

If either the target VAL UE or the reference VAL UE lost connectivity, the location management server determines the target VAL UE is outside the predetermined area of interest or dynamic area of interest as follows.

-	If the target VAL UE lost connectivity, and the target VAL UE's current absolute location is obtained in step 6 or step 7, the LMS server determines the target VAL UE is outside the predetermined area of interest according to the target VAL UE's absolute location. The location management server determines the target VAL UE is not within the proximity range of the reference VAL UE based on the reference UEs' location obtained in step 4 and the target VAL UE's absolute location in step 6 or step 7.

-	If the target VAL UE lost connectivity, and target VAL UE's current relative location is obtained in step 6, the LM server determines the target VAL UE is not in the predetermined area of interest or not within the proximity range of the reference VAL UE based on the reference VAL UEs' location obtained in step 4 and the target VAL UE's absolute location in step 6 or step 7.

-	If the reference VAL UE lost connectivity, and the reference VAL UE's absolute location or relative location is obtained instep 6 or step 7, the location management server determines the target VAL UE is not within the proximity range of the reference VAL UE based on the reference VAL UEs' location obtained in step 4 and the target VAL UE's absolute location in step 6 or step 7.

The LM server notifies the VAL server "Notify Absence" message.

9c.	When the target VAL UE's current location is either in the predetermined area of interest or in the proximity range of the reference UE, and such in area of interest result is also aligned with the NEF/GMLC reported area of interest monitoring result, then the LM server shall notify ("Notify Presence" message) the VAL server periodically, according to the "Notify_Interval" value in "Monitor Location Subscription Request" message, indicating the VAL server that the target VAL UE is within the area of interest, along with target VAL UE's current location information.

If either the target VAL UE or the reference VAL UE lost connectivity, the LM server determines the target VAL UE is within the predetermined area of interest or dynamic area of interest as follows.

-	If the target VAL UE lost connectivity, and the target VAL UE's current absolute location is obtained in step 6 or step 7, the LMS server determines the target VAL UE is within the predetermined area of interest according to the target VALUE’s absolute location. The LM server determines the target VAL UE is within the proximity range of the reference VAL UE based on the reference VAL UEs' location obtained in step 4 and the target VAL UE’s absolute location in step 6 or step 7.

-	If the target VAL UE lost connectivity, and target VAL UE's current relative location is obtained in step 6 or step 7, the LM server determines the target VAL UE is within the predetermined area of interest or within the proximity range of the reference VAL UE based on the reference VAL UEs’ location obtained in step 4 and the target UE's absolute location in step 6 or step 7.

-	If the reference VAL UE lost connectivity, and the reference VAL UE's absolute location or relative location is obtained in step 6 or step 7, the LM server determines the target VAL UE is within the proximity range of the reference VAL UE based on the reference VAL UEs’ location obtained in step 4 and the target VAL UE's absolute location in step 6 or step 7.

And the location management server shall notify the VAL server "Notify Presence" message.

9d.	If the location management server is notified by the 3GPP CN about loss of connectivity for the reference VAL UE and target VAL UE(s), or fails to obtains the target VAL UE's location or the reference UE' location in step 6 or step 7 (e.g., due to the target VAL UE and reference VAL UE are beyond the Ranging/SL range), then the location management server notifies ("Notify Unknown" message with detailed reason) the VAL server indicating that the location management server does not know whether the target VAL UE is "Presence" or “Absence", therefore the dynamic geofencing monitoring for the impacted target UE(s) is suspended.

NOTE 5:	Upon "Notify Unknown", the VAL server can remind end user to check status for the reference/target UE, which is out of 3GPP scope.

Editor's Note:	Detailed handling for “loss of connectivity” in LM server is FFS. For instance, whether to address the case when reference UE and target UE(s) both lose connectivity (e.g., using UE-only operation for SL/Ranging).

NOTE 6:	Notifications in step 9 can be subject to explicit notification indication from the VAL server, for instance, the VAL server can be interested to know only "Notify Absence" and "Notify Unknown".

10.	If both the target VAL UE and/or reference VAL UE lost the connectivity, the off-network target VAL UE and/or reference VAL UE initiate the UE-only Operation SL/Ranging as described in 3GPP TS 23.586 [57] or the procedure as defined in clause 9.5.3 and clause 9.5.4, and cache the location result as configured in step 4e.

11. If the connectivity is back, the target VAL UE and/or reference VAL UE check whether it is still within the valid duration if configured. If yes, the the target UE and/or reference VAL UE report the cached history location to the location management server as described in clause 9.3.21.3.1.

12.	The location management server may use the cached history location e.g., to notify the VAL server about the history target VAL UE location status during the loss of connectivity.

****************************End of changes*******************************************

##### 8.1.4.1.5	Changes to cluse 9.3.14 of 3GPP TS 23.434 [3]

The changes to Clause 9.3.7 of 3GPP TS 23.434 [3] should be changed as the following (new text showed in the highighed text).

******************************Begin of Changes***************************************************

9.3.26.3	Confirm location verification

The location management client (LMC)/application in the UE gets from non-3GPP positioning (e.g. GNSS) in the UE its location. The application in the UE wants to check if this is really the correct location.

Pre-conditions:

1.	The LMC is authorized to discover and to use Confirm Location API provided by the LMS;

2.	LMS is authorized to use Nnef Event Exposure API, based on SLA with NEF/5GS of MNO, or Ngmlc_Location_EventNotify API;

3.	It is assumed there is user consent given in the UE for exposing location information from UE to LMS.

Figure 9.3.26.3-1: Procedure of confirm location verification

1.	The UE sends Confirm Location request from the LMC towards LMS. It contains the location measured from the UE (non-3GPP location information, e.g. GNSS). It contains latitude and longitude, accuracy, timestamp, carrier name, cell id (known to the UE from the network signalling), application identifier.

2.	 The LMS shall check if there is subscription (based on VAL subscription in clause 6.12.2.1) for confirm location service with the specific application identifier received in step1. If subscription is present, step 3 is performed. If subscription is not present the LMS replies back to LMC with confirm location status unknown (step 4).

3.	LMS requests UE location from 5GC/LMF via NEF (LMS send determine Location operation via NEF (N33 interface) and using Nlmf_Location API defined in 3GPP TS 29.572 [8] – parameters available from the location reporting are described in the TS). It also requests accuracy to be a parameter present in the response. LMS can also perform a location information request to GMLC directly or via NEF (as defined in 6.1. of 3GPP TS 23.273 [5]), acting as AF. LMS can also perform a location information request to 3rd party location servers via LM-3P interface. LMS can also use the T8 interface to obtain UE location. Or the LMS can have already the UE location based on the methods described in 3GPP TS 23.434 [9]. The received response contains the UE location from the network. LMS compares the location provided from the network with the location provided from the LMC. LMS can use event exposure using NEF according to table 4.15.3.1-1 of TS 23.502 [11] to obtain user location information from AMF.

4.	LMS sends confirm location report to LMC.

5.	The LMS sends notification towards the VAL server with the UE identifier and application using the service.

#### 8.1.4.2	Solution evaluation

This solution well addresses the solution about how to address LMS related content for Gap#4-3-2-1. It makes an aligned description for the case where LMS is deployed with trusted domain of MNO.

### 8.1.5	Technical Solution#5: Align Security Credential with 3GPP TS 33.434 [13] and Clean Application Enablement Specifications

#### 8.1.5.1	Solution description

This solution maps to Gaps in clause 4.5. This solution is about how to align Security Credential with 3GPP TS 33.434[13] and clean application enablement specifications.

Security credential(s) for authorization and verification as well as security mechanisms for SEAL services in general are specified in 3GPP TS 33.434[13]. However, the term Security Credential was added in many SEAL service procedures and information flows, but without definition.

Based on the defined security mechanisms in 3GPP TS 33.434 [13], there can be different ways to exchange security information, whether in the form of credentials or some other information used as input in the security procedures. Adding opaque "security credentials" element in many of the SEAL information flows does not always match the security mechanism specified in 3GPP TS 33.434 13] and it is not helpful given that the actual security related data is defined elsewhere than in application enablement services.

To avoid including unusual information in the SEAL information flows, which would be over-written with procedures specific data in 3GPP TS 33.434 [13] and by protocol-specific security data at stage 3 implementation, it is recommended to consistently correct this misalignment throughout the SEAL information flows.

Since the inclusion of an opaque element Security Credential in SEAL information flows of application enablement specifications does not clarify nor provide any support with the actual security data included and used for security procedures, this solution proposes for application enablement specifications in Rel-20 and future releases:

-	avoid adding the term "Security Credential" to procedure and information flows, etc.; and

-	add separate clause for security, with general description on security aspects to set up link with 3GPP TS 33.434 [13].

#### 8.1.5.2	Solution evaluation

The solution provides a method to avoid using undefined parameter in SEAL specifications and avoid misalignment across different WGs.

### 8.1.6	Technical Solution #6: Multiple NRM services for network service QoS control

#### 8.1.6.1	Solution description

This solution proposes to address the gaps identified in clause 4.1 regarding the support various application requirements at NRM.

It is proposed to introduce a new set of NRM services to support the various applications per their requirements to the network resource and handling based on the following principles:

-	The new NRM services are designed per application layer requirements as described in clause 8.4.A.

-	The input parameters of each service API are avoiding excessive configurations and necessary to achieve the desired benefit corresponding to the application requirement(s).

-	The name of the service, service APIs, input parameters are familiar to the application community.

The list of several NRM services are as follows:

-	QoS control service for network transport with high data rate and low latency. This service enables the VAL server to request the network transport resource with high data rate and low latency. This may be used by AR/VR applications.

-	QoS control service for network transport with low latency. This service enables the VAL server to request network transport resource with low latency. Optionally, the loose requirements on bit rate are applied. This may be used by multi-modal communications.

-	QoS control service for network transport with low latency and high reliability. This service enables the VAL server to request network transport resource with low latency and high reliability. This may be used by cyber-physical control applications, V2X, rail communications.

-	QoS control service for network transport with low latency & high bandwidth & high reliability. This service enables the VAL server to request network transport resource with low latency, high bandwidth and high reliability. This may be used by AV production service, medical or gaming applications.

The new set of QoS control services at the NRM is aiming at the application stakeholders who lack the 3GPP network background but wish to start an agile development in a short cycle.

##### 8.1.6.1.1	Procedures

###### 8.1.6.1.1.1		QoS control service for network transport with high data rate and low latency service

The figure 8.1.6.1.1.1-1 illustrates the high-level procedures of creating the QoS control service for network transport with high data rate and low latency service.

Pre-conditions:

The Nrm_QosCtrlHighDataRateLowLatency service has been discovered by the ASP and programmed into the VAL server.

Figure 8.1.6.1.1.1-1: Create QoS control service for network transport with high data rate and low latency

1.	The VAL server invokes the Nrm_QosCtrlHighDataRateLowLatency create service to request the network transport resource with high data rate and low latency QoS guarantee. The high data rate requirements (e.g., requested maximum bitrate, requested guaranteed bitrate, media type, media format, bandwidth), low latency requirements (e.g., UL, DL and/or round-trip latency) are included. The service identifier which the high data rate requirements and the low latency requirements are applied and the UE ID are also included.

2.	Based on the request in step 1, the NRM server perform authorization check. If authorization is successful, the NRM server invokes the Nef_AFsessionwithQoS_create request (if the NRM server is owned by 3rd party and outside the MNO trust domain) or Npcf_PolicyAuthorization_create request (if the NRM server is within MNO trust domain). The NRM server selects the corresponding features and set the input parameters of the network service based on the parameters received in step 1. The 3GPP network processes the service invocation from the NRM and returns the Nef_AFsessionwithQoS_create response and the Npcf_PolicyAuthorization_create response.

3.	The NRM server returns the Nrm_QosCtrlHighDataRateLowLatency create response indication the result.

The figure 8.1.6.1.1.1-2 illustrates the high-level procedures of updating the QoS control service for network transport with high data rate and low latency service.

Pre-conditions:

The Nrm_QosCtrlHighDataRateLowLatency service has been discovered by the ASP and programmed into the VAL server.

Figure 8.1.6.1.1.1-2: Update QoS control service for network transport with high data rate and low latency

1.	The VAL server invokes the Nrm_QosCtrlHighDataRateLowLatency update service to update the network transport resource with high data rate and low latency QoS guarantee. The updated high data rate requirements and/or updated low latency requirements are also included.

2.	Based on the request in step 1, the NRM server perform authorization check. If authorization is successful, the NRM server invokes the Nef_AFsessionwithQoS_update request (if the NRM server is owned by 3rd party and outside the MNO trust domain) or Npcf_PolicyAuthorization_update request (if the NRM server is within MNO trust domain). The NRM server selects the corresponding features and set the input parameters of the network service based on the parameters received in step 1. The 3GPP network processes the service invocation from the NRM and returns the Nef_AFsessionwithQoS_update response and the Npcf_PolicyAuthorization_update response.

3.	The NRM server returns the Nrm_QosCtrlHighDataRateLowLatency update response indication the result.

The figure 8.1.6.1.1.1-3 illustrates the high-level procedures of deleting the QoS control service for network transport with high data rate and low latency service.

Pre-conditions:

The Nrm_QosCtrlHighDataRateLowLatency service has been discovered by the ASP and programmed into the VAL server.

Figure 8.1.6.1.1.1-3: Delete QoS control service for network transport with high data rate and low latency

1.	The VAL server invokes the Nrm_QosCtrlHighDataRateLowLatency delete service to delete the network transport resource with high data rate and low latency service.

2.	Based on the request in step 1, the NRM server perform authorization check. If authorization is successful, the NRM server invokes the Nef_AFsessionwithQoS_delete request (if the NRM server is owned by 3rd party and outside the MNO trust domain) or Npcf_PolicyAuthorization_delete request (if the NRM server is within MNO trust domain). The NRM server selects the corresponding features and set the input parameters of the network service based on the parameters received in step 1. The 3GPP network processes the service invocation from the NRM and returns the Nef_AFsessionwithQoS_delete response and the Npcf_PolicyAuthorization_delete response.

3.	The NRM server returns the Nrm_QosCtrlHighDataRateLowLatency delete response indication the result.

###### 8.1.6.1.1.2	QoS control service for network transport with low latency service

The figure 8.1.6.1.1.2-1 illustrates the high-level procedures of creating the QoS control service for network transport with low latency service.

Pre-conditions:

The Nrm_QosCtrlLowLatency service has been discovered by the ASP and programmed into the VAL server.

Figure 8.1.6.1.1.2-1: Create QoS control service for network transport with low latency

1.	The VAL server invokes the Nrm_QosCtrlLowLatency create service to request the network transport resource with low latency QoS guarantee. The low latency requirements (e.g., UL, DL and/or round-trip latency) are included. The service identifier which the high data rate requirements and the low latency requirements are applied and the UE ID are also included.

2.	Based on the request in step 1, the NRM server perform authorization check. If authorization is successful, the NRM server invokes the Nef_AFsessionwithQoS_create request (if the NRM server is owned by 3rd party and outside the MNO trust domain) or Npcf_PolicyAuthorization_create request (if the NRM server is within MNO trust domain). The NRM server selects the corresponding features and set the input parameters of the network service based on the parameters received in step 1. The 3GPP network processes the service invocation from the NRM and returns the Nef_AFsessionwithQoS_create response and the Npcf_PolicyAuthorization_create response.

3.	The NRM server returns the Nrm_QosCtrlLowLatency create response indication the result.

The figure 8.1.6.1.1.2-2 illustrates the high-level procedures of updating the QoS control service for network transport with high data rate and low latency service.

Pre-conditions:

The Nrm_QosCtrlLowLatency service has been discovered by the ASP and programmed into the VAL server.

Figure 8.1.6.1.1.2-2: Update QoS control service for network transport with low latency

1.	The VAL server invokes the Nrm_QosCtrlLowLatency update service to update the network transport resource with low latency QoS guarantee. The updated low latency requirements are also included.

2.	Based on the request in step 1, the NRM server perform authorization check. If authorization is successful, the NRM server invokes the Nef_AFsessionwithQoS_update request (if the NRM server is owned by 3rd party and outside the MNO trust domain) or Npcf_PolicyAuthorization_update request (if the NRM server is within MNO trust domain). The NRM server selects the corresponding features and set the input parameters of the network service based on the parameters received in step 1. The 3GPP network processes the service invocation from the NRM and returns the Nef_AFsessionwithQoS_update response and the Npcf_PolicyAuthorization_update response.

3.	The NRM server returns the Nrm_QosCtrlLowLatency update response indication the result.

The figure 8.1.6.1.1.2-3 illustrates the high-level procedures of deleting the QoS control service for network transport with low latency service.

Pre-conditions:

The Nrm_QosCtrlLowLatency service has been discovered by the ASP and programmed into the VAL server.

Figure 8.1.6.1.1-3: Delete QoS control service for network transport with low latency

1.	The VAL server invokes the Nrm_QosCtrlLowLatency delete service to delete the network transport resource with low latency service.

2.	Based on the request in step 1, the NRM server perform authorization check. If authorization is successful, the NRM server invokes the Nef_AFsessionwithQoS_delete request (if the NRM server is owned by 3rd party and outside the MNO trust domain) or Npcf_PolicyAuthorization_delete request (if the NRM server is within MNO trust domain). The NRM server selects the corresponding features and set the input parameters of the network service based on the parameters received in step 1. The 3GPP network processes the service invocation from the NRM and returns the Nef_AFsessionwithQoS_delete response and the Npcf_PolicyAuthorization_delete response.

3.	The NRM server returns the Nrm_QosCtrlLowLatency delete response indication the result.

###### 8.1.6.1.1.3	QoS control service for network transport with low latency and high reliability service

The figure 8.1.6.1.1.3-1 illustrates the high-level procedures of creating the QoS control service for network transport with low latency and high reliability service.

Pre-conditions:

The Nrm_QosCtrlLowLatencyHighReliability service has been discovered by the ASP and programmed into the VAL server.

Figure 8.1.6.1.1.3-1: Create QoS control service for network transport with low latency and high reliability

1.	The VAL server invokes the Nrm_QoSCtrlLowLatencyHighReliability create service to request the network transport resource with low latency and high reliability QoS guarantee. The low latency requirements (e.g., UL, DL and/or round-trip latency) and high reliability requirements (e.g., request packet error rate) are included. The service identifier which the high data rate requirements and the low latency requirements are applied and the UE ID are also included.

2.	Based on the request in step 1, the NRM server perform authorization check. If authorization is successful, the NRM server invokes the Nef_AFsessionwithQoS_create request (if the NRM server is owned by 3rd party and outside the MNO trust domain) or Npcf_PolicyAuthorization_create request (if the NRM server is within MNO trust domain). The NRM server selects the corresponding features and set the input parameters of the network service based on the parameters received in step 1. The 3GPP network processes the service invocation from the NRM and returns the Nef_AFsessionwithQoS_create response and the Npcf_PolicyAuthorization_create response.

3.	The NRM server returns the Nrm_QoSCtrlLowLatencyHighReliability create response indication the result.

The figure 8.1.6.1.1.3-2 illustrates the high-level procedures of updating the QoS control service for network transport with low latency and high reliability service.

Pre-conditions:

The Nrm_QoSCtrlLowLatencyHighReliability service has been discovered by the ASP and programmed into the VAL server.

Figure 8.1.6.1.1.2-2: Update QoS control service for network transport with low latency and high reliability

1.	The VAL server invokes the Nrm_QoSCtrlLowLatencyHighReliability update service to request the network transport resource with low latency and high reliability QoS guarantee. The updated low latency requirements and/or updated high reliability requirements are also included.

2.	Based on the request in step 1, the NRM server perform authorization check. If authorization is successful, the NRM server invokes the Nef_AFsessionwithQoS_update request (if the NRM server is owned by 3rd party and outside the MNO trust domain) or Npcf_PolicyAuthorization_update request (if the NRM server is within MNO trust domain). The NRM server selects the corresponding features and set the input parameters of the network service based on the parameters received in step 1. The 3GPP network processes the service invocation from the NRM and returns the Nef_AFsessionwithQoS_update response and the Npcf_PolicyAuthorization_update response.

3.	The NRM server returns the Nrm_QoSCtrlLowLatencyHighReliability update response indication the result.

The figure 8.1.6.1.1.3-3 illustrates the high-level procedures of deleting the QoS control service for network transport with low latency and high reliability service.

Pre-conditions:

The Nrm_QoSCtrlLowLatencyHighReliability service has been discovered by the ASP and programmed into the VAL server.

Figure 8.1.6.1.1.3-3: Delete QoS control service for network transport with low latency and high reliability

1.	The VAL server invokes the Nrm_QoSCtrlLowLatencyHighReliability delete service to delete the network transport resource with low latency and high reliability service.

2.	Based on the request in step 1, the NRM server perform authorization check. If authorization is successful, the NRM server invokes the Nef_AFsessionwithQoS_delete request (if the NRM server is owned by 3rd party and outside the MNO trust domain) or Npcf_PolicyAuthorization_delete request (if the NRM server is within MNO trust domain). The NRM server selects the corresponding features and set the input parameters of the network service based on the parameters received in step 1. The 3GPP network processes the service invocation from the NRM and returns the Nef_AFsessionwithQoS_delete response and the Npcf_PolicyAuthorization_delete response.

3.	The NRM server returns the Nrm_QoSCtrlLowLatencyHighReliability delete response indication the result.

###### 8.1.6.1.1.4	QoS control service for network transport with low latency & high bandwidth & high reliability service

The figure 8.1.6.1.1.4-1 illustrates the high-level procedures of creating QoS control service for network transport with low latency & high bandwidth & high reliability service.

Pre-conditions:

The Nrm_QosCtrlLowLatencyHighBWHighReliability service has been discovered by the ASP and programmed into the VAL server.

Figure 8.1.6.1.1.4-1: Create QoS control service for network transport with low latency & high bandwidth & high reliability

1.	The VAL server invokes the Nrm_QosCtrlLowLatencyHighBWHighReliability create service to request the network transport resource with low latency and high bandwidth and high reliability QoS guarantee. The low latency requirements, high bandwidth requirements (e.g., requested maximum bitrate, requested guaranteed bitrate, media type, media format, bandwidth) and the high reliability requirements are included. The service identifier which the high data rate requirements and the low latency requirements are applied and the UE ID are also included.

2.	Based on the request in step 1, the NRM server perform authorization check. If authorization is successful, the NRM server invokes the Nef_AFsessionwithQoS_create request (if the NRM server is owned by 3rd party and outside the MNO trust domain) or Npcf_PolicyAuthorization_create request (if the NRM server is within MNO trust domain). The NRM server selects the corresponding features and set the input parameters of the network service based on the parameters received in step 1. The 3GPP network processes the service invocation from the NRM and returns the Nef_AFsessionwithQoS_create response and the Npcf_PolicyAuthorization_create response.

3.	The NRM server returns the Nrm_QosCtrlLowLatencyHighBWHighReliability create response indication the result.

The figure 8.1.6.1.1.1-2 illustrates the high-level procedures of updating QoS control service for network transport with low latency & high bandwidth & high reliability service.

Pre-conditions:

The Nrm_QosCtrlLowLatencyHighBWHighReliability service has been discovered by the ASP and programmed into the VAL server.

Figure 8.1.6.1.1.4-2: Update QoS control service for network transport with low latency & high bandwidth & high reliability

1.	The VAL server invokes the Nrm_QosCtrlLowLatencyHighBWHighReliability update service to update the network transport resource with low latency and high bandwidth and high reliability service. The updated low latency requirements, updated high bandwidth requirements, and/or updated high reliability requirements are also included.

2.	Based on the request in step 1, the NRM server perform authorization check. If authorization is successful, the NRM server invokes the Nef_AFsessionwithQoS_update request (if the NRM server is owned by 3rd party and outside the MNO trust domain) or Npcf_PolicyAuthorization_update request (if the NRM server is within MNO trust domain). The NRM server selects the corresponding features and set the input parameters of the network service based on the parameters received in step 1. The 3GPP network processes the service invocation from the NRM and returns the Nef_AFsessionwithQoS_update response and the Npcf_PolicyAuthorization_update response.

3.	The NRM server returns the Nrm_QosCtrlLowLatencyHighBWHighReliability update response indication the result.

The figure 8.1.6.1.1.4-3 illustrates the high-level procedures of deleting the QoS control service for network transport with low latency & high bandwidth & high reliability service.

Pre-conditions:

The Nrm_QosCtrlLowLatencyHighBWHighReliability service has been discovered by the ASP and programmed into the VAL server.

Figure 8.1.6.1.1.4-3: Delete QoS control service for network transport with low latency & high bandwidth & high reliability

1.	The VAL server invokes the Nrm_QosCtrlLowLatencyHighBWHighReliability delete service to delete the network transport resource with low latency and high bandwidth and high reliability service.

2.	Based on the request in step 1, the NRM server perform authorization check. If authorization is successful, the NRM server invokes the Nef_AFsessionwithQoS_delete request (if the NRM server is owned by 3rd party and outside the MNO trust domain) or Npcf_PolicyAuthorization_delete request (if the NRM server is within MNO trust domain). The NRM server selects the corresponding features and set the input parameters of the network service based on the parameters received in step 1. The 3GPP network processes the service invocation from the NRM and returns the Nef_AFsessionwithQoS_delete response and the Npcf_PolicyAuthorization_delete response.

3.	The NRM server returns the Nrm_QosCtrlLowLatencyHighBWHighReliability delete response indication the result.

###### 8.1.6.1.1.5	Information flow

Table 8.1.6.1.1.5-1 describes the information flow Nrm_QosCtrlHighDataRateLowLatency from the consumer to the NRM server.

Table 8.1.6.1.1.5-1: Nrm_QosCtrlHighDataRateLowLatency create request

| Information element | Status | Description |
| --- | --- | --- |
| Service identifier | M | The identity of the application traffic for whom the network resource QoS control is requested |
| VAL UE ID(s) | M | One or more VAL UE IDs for whom the network resource is requested |
| High data rate requirements | M | The requirement description of high data rate (e.g., requested maximum bitrate, requested guaranteed bitrate, media type, media format, bandwidth) |
| Low latency requirements | M | The requirement description of low latency (e.g., UL, DL and/or round-trip latency) |

Table 8.1.6.1.1.5-2 describes the information flow Nrm_QosCtrlHighDataRateLowLatency create response from the NRM server to the consumer.

Table 8.1.6.1.1.5-2: Nrm_QosCtrlHighDataRateLowLatency create response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the network resource reservation with the underlying network. |

Table 8.1.6.1.1.5-3 describes the information flow Nrm_QosCtrlLowLatency create request from consumer to the NRM server.

Table 8.1.6.1.1.5-3: Nrm_QosCtrlLowLatency create request

| Information element | Status | Description |
| --- | --- | --- |
| Service identifier | M | The identity of the application traffic for whom the network resource QoS control is requested |
| VAL UE ID(s) | M | One or more VAL UE IDs for whom the network resource is requested |
| Low latency requirements | M | The requirement description of low latency (e.g., UL, DL and/or round-trip latency) |

Table 8.1.6.1.1.5-4 describes the information flow Nrm_QosCtrlLowLatency create response from NRM server to the consumer.

Table 8.1.6.1.1.5-4: Nrm_QosCtrlLowLatency create response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the network resource reservation with the underlying network. |

Table 8.1.6.1.1.5-5 describes the information flow Nrm_QosCtrlLowLatencyHighReliability from the consumer to the NRM server.

Table 8.1.6.1.1.5-5: Nrm_QosCtrlLowLatencyHighReliability create request

| Information element | Status | Description |
| --- | --- | --- |
| Service identifier | M | The identity of the application traffic for whom the network resource QoS control is requested |
| VAL UE ID(s) | M | One or more VAL UE IDs for whom the network resource is requested |
| Low latency requirements | M | The requirement description of low latency (e.g., UL, DL and/or round-trip latency) |
| High reliability | M | The requirement description of high reliability (e.g., request packet error rate) |

Table 8.1.6.1.1.5-6 describes the information flow Nrm_QosCtrlLowLatencyHighReliability create response from the NRM server to the consumer.

Table 8.1.6.1.1.5-6: Nrm_QosCtrlLowLatencyHighReliability create response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the network resource reservation with the underlying network. |

Table 8.1.6.1.1.5-7 describes the information flow Nrm_QosCtrlLowLatencyHighBWHighReliability from the consumer to the NRM server.

Table 8.1.6.1.1.5-1: Nrm_QosCtrlLowLatencyHighBWHighReliability create request

| Information element | Status | Description |
| --- | --- | --- |
| Service identifier | M | The identity of the application traffic for whom the network resource QoS control is requested |
| VAL UE ID(s) | M | One or more VAL UE IDs for whom the network resource is requested |
| Low latency requirements | M | The requirement description of low latency (e.g., UL, DL and/or round-trip latency) |
| High bandwidth requirements | M | The requirement description of high bandwidth (e.g., requested maximum bitrate, requested guaranteed bitrate, media type, media format, bandwidth) |
| High reliability | M | The requirement description of high reliability (e.g., request packet error rate) |

Table 8.1.6.1.1.5-8 describes the information flow Nrm_QosCtrlLowLatencyHighBWHighReliability create response from the NRM server to the consumer.

Table 8.1.6.1.1.5-8: Nrm_QosCtrlLowLatencyHighBWHighReliability create response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the network resource reservation with the underlying network. |

#### 8.1.6.2	NRM services

The following new NRM service listed in table 8.a.1.3 are introduced:

Table 8.a.1.3-1: List of new NRM services for network resource control

| Service Name | Known Consumer(s) | Communication Type |
| --- | --- | --- |
| QoS control service for network transport with high data rate and low latency | VAL | Request/response |
| QoS control service for network transport with low latency | VAL | Request/response |
| QoS control service for network transport with low latency and high reliability | VAL | Request/response |
| QoS control service for network transport with low latency & bandwidth & high reliability | VAL | Request/response |

#### 8.1.6.3 	NRM services API

Editor's note:	The solution based on a single merged API or multiple APIs to support different services, and the evaluation of the multi-API and single-API solutions, are FFS

#### 8.1.6.4	Solution evaluation

This solution provides several new NRM services for network service QoS control to the application service providers/developers. Each of the new NRM service represents a different QoS service catalog and allow the application stakeholders to choose a proper QoS control service corresponding to their application requirements.

### 8.1.7	Solution#7: Enhancements of NRM network resource adaptation service

#### 8.1.7.1	Solution Description

This solution addresses technical gap#1 in clause 4.1.

It is important for the NRM services to provide sufficient input parameters to support latest network capabilities such as multi-modal application traffic. The existing NRM network resource adaptation service can be enhanced for multi-modal traffic by providing additional input parameters that enables the application client/server to request network resource adaptation by specifying the traffic type along with the priority of the traffics of the application and the QoS requirements for each traffic type of the application.

The application server sends the network resource adaptation request with resource adaptation requirements to the NRM server. The resource adaptation requirements are with input parameters specifying the traffic type(s) e.g., audio, video, sensor data along with the priority of the traffic flows.

In some cases, the application may need to indicate the priority of one traffic flow compared to other proposed traffic flows.

The enhanced NRM network resource adaptation service for multi-modal traffic allows the application client/server to add the QoS requirements for each traffic types separately. For example, consider the above mentioned gaming application, QoS requirements such as high reliablility and low latency can be requested for the "video" traffic type and high data rate and low latency can be requested for the "haptic" traffic type.

The enhanced NRM network resource adaptation service for multi-modal traffic can send traffic type specific QoS requirements to the 5GC and can be mapped with the AFSessionWithQoS for multi-modal service. The multi-modal service requirements information for each data flow can be mapped with the QoS requirements of different traffic types.

The proposed input parameters to the enhanced NRM network resource adaptation service for multi-modal traffic makes it suitable for scenarios with single or multiple traffic types, irrespective of the application type.

The following clauses specify the procedures and information flows for gaps identified in clause 4.1 to support the enhancements of the NRM network resource adaptation service for multi-modal traffic.

##### 8.1.7.2.1	Impact to existing NRM information flows

The information flows of NRM network resource adaptation service in 3GPP TS 23.434 [3] can be enhanced (highlighted in bold italics) as follows.

14.3.3.3.1 Network resource adaptation ###### 14.3.3.3.1.1 General This subclause describes the procedure for network resource adaptation using PCC procedures. This procedure satisfies the requirements for requesting unicast resources and modification to already allocated unicast resources to VAL communications. ###### 14.3.3.3.1.2 Procedure Figure 14.3.3.3.1.2-1 illustrates the procedure for the network resource adaptation. Figure 14.3.3.3.1.2-1: Network resource adaptation - The VAL server (or SEALDD server) sends a network resource adaptation request to the NRM server for one or more users belonging to one or more VAL services, and may comprise one or more VAL UEs that will have updated resource requirement. This requirement may be in the form of exact resources /resource pools to be used or indication of bandwidth increase/decrease for the corresponding VAL UEs or set of VAL UEs. The BDT Reference ID may also be included for setting BDT policies. The resource adaptation requirement(s) IE in the request may include traffic type to indicate the different types of traffic flows involved (for e.g. audio, video, haptic), priority of the traffic types (the VAL server can set the priority of the traffic types involved using the priority IE), QoS requirement per traffic type. NOTE: The SEALDD server can use the NRM netwok resource adaption service as per the procedure defined in 3GPP TS 23.433 clause 9.11 SEALDD Background data transfer. 2. The NRM server processes the request and applies / enforces the resource adaptation per VAL UE. 3. The NRM server retrieves UE IP address by using event monitoring capability for PDU session status (or PDN connectivity status) and initiates the PCC procedures for the traffic flows associated to each VAL UE. Editor NOTE 1: It is FFS if UE IP address can be sent from VAL server. Editor NOTE 2: Whether and how multi-modal service ID needs to be got by NRM server, and used in this step. 4. The NRM server provides a network resource adaptation response to the VAL server, providing information on the fulfilment of the network resource adaptation request. This will include information either per VAL UE or per set of VAL UEs, as indicated by the request of the VAL server in step 1. If the QoS requirement for traffic type(s) cannot be satisfied, the cause of failure may be included in the response. ###### 14.3.2 Information flows #### 14.3.2.1 Network resource adaptation request Table 14.3.2.1-1 describes the information flow network resource adaptation request from the VAL/SEALDD server to the NRM server. Table 14.3.2.1-1: Network resource adaptation request Information element; Status; Description. Requester Identity; M; The identity of the VAL server performing the request.. List of VAL UE IDs; O (see NOTE 1); List consisting of one or more VAL UE IDs for whom the network resource adaptation occurs.. VAL group ID; O (see NOTE 1); The VAL group ID for whom the network resource adaptation occurs.. Resource adaptation requirement(s); M; The resource adaptation requirement corresponds to the VAL service QoS requirements as applied for a UE or group of UEs (E.g. bandwidth, resource).. >Traffic type; O; The types of the traffic flows involved (e.g, audio, video, haptics etc).. >Priority; O (see NOTE 2); Indicates the priority of the traffic type.. >QoS requirement; M; The QoS requirement corresponds to the VAL service QoS requirement as per the traffic type (e.g, data rate, GBR, MBR, latency, reliability, bandwidth, etc., and/or its combinations).. BDT Reference ID; O; Indicates the Background data transfer Reference ID provided by the 3GPP network. NOTE 1: Either of the information elements should be present. NOTE 2: Priority may be present only when the Traffic type is present.. #### 14.3.2.2 Network resource adaptation response Table 14.3.2.2-1 describes the information flow network resource adaptation response from the NRM server to the VAL server. Table 14.3.2.2-1: Network resource adaptation response Information element; Status; Description. Result; M; Result includes success or failure of the network resource adaptation with the underlying network. The response can also include an updated value for some of the parameters included in the network resource adaptation request (e.g. negotiation of resource offering). Cause; O (see NOTE); Cause of failure (e.g. QoS requirement for traffic type cannot be satisfied).. NOTE: The information element shall be present only when the result is failure..

#### 8.1.7.2	Solution evaluation

This solution provide mechanism to enable NRM to support network resource adaptation service for multi-modal applications. There are two some open issues for UE IP address and multi-modal service ID in solution.

It has no architecture impact.

### 8.1.8	Technical solution #8: Representation of SEAL service consumer

#### 8.1.8.1	Solution description

This solution addresses technical gap#6 in clause 4.6.

The following sentences for clarification should be added into 3GPP TS 23.434[3], 3GPP TS23.435[4], 3GPP TS23.436[5], 3GPP TS23.437[6], 3GPP TS23.438[7], 3GPP TS23.482[12]:

In present document, there is no general restriction on the type of potential consumers of SEAL services if SEAL service provider has no special restriction policy on certain services. Any SEAL service consumer can use the SEAL services by applying to the required implementation on CAPIF-1/1e, CAPIF2/2e.

VAL server in this document is showed as one example of SEAL service consumers.

#### 8.1.8.2	Solution evaluation

This solution removes unnecessary restriction on the consumers of SEAL services. It makes SEAL service specification aligned with application implementation mechanisms, and improves the value of SEAL by allowing more customers.

### 8.1.9	Technical solution #9: New NRM service for IoT power saving

#### 8.1.9.1	Solution description

This solution proposes to address the gaps identified in clause 4.4 regarding the support power saving requirements at NRM. based on the following principles:

- The new NRM services are designed per application layer requirements as described in clause 8.4.A single service API serves a single service;

- The input parameters of each service are minimum and necessary to achieve the desired benefit corresponding to the application requirement(s);

- The name of the service, service APIs, input parameters are familiar to the applications.

The new NRM service is as follow:

-	IoT Power saving service. This service enables the VAL server to provide the application traffic characteristics e.g., application traffic patterns to the network for power saving setting. At the same time, it allows the VAL server to subscribe the power saving related information e.g., UE idle status, maximum UE availability time from the network.

This new service is aiming at the IoT service providers/developers who lack the 3GPP network background and wish to start an agile development in a short cycle.

##### 8.1.9.1.1	Procedures

The figure 8.9.1.1-1 illustrates the high-level procedure for the application layer consumer (e.g., VAL server) to use the power saving service.

Figure 8.9.1.1-1: IoT Power saving invocation procedure

1.	The application layer consumer (e.g., VAL server) invokes the IoT power saving service. The input parameters e.g., UE identity, external group identity, application communication patterns parameters (e.g., duration time of periodic communication, interval time of periodic communication, time zone and day of the week when the UE is available for communication), suggested network parameters (e.g., maximum delay acceptable for downlink data transfers, the length of time for which the UE stays reachable to allow the SCS/AS to reliably deliver the required downlink data, the number of packets that the serving gateway shall buffer in case that the UE is not reachable.), power saving event subscription indication are provided.

2. Upon receiving the service request, the NRM determines to invoke one or more network related APIs addressing the power saving aspects.

3. The NRM server may invoke one or more network related APIs (either via the NEF/SCEF or directly towards the NF(s) which provides the corresponding service) listed below:

3a.	If the power saving event subscription indication is included in step 1, the NRM server invokes the MonitorEvent API as defined in clause 5.3 of 3GPP TS 29.122 [15] to subscribe idle status information.

3b.	If the application communication patterns parameters are included, the NRM server invokes the CpProvisioning API as defined in clause 5.10 of 3GPP TS 29.122 [15] to enable the network resource optimizations.

3c.	If the suggested network parameters are included, NpProvisioning API as defined in clause 5.13 of 3GPP TS 29.122 [15 to influence certain aspects of UE/network behaviour such as the UE's PSM, extended idle mode DRX, and extended buffering configurations.

4.	The NRM server returns a IoT power saving service response. This may be sent before step 3.

5.	Optionally, the NRM server further returns a IoT power saving service notification with the idle status information as described in clause 5.3.2.3.3 of 3GPPP TS 29.122 [15]. The VAL server further utilizes the information to control the data exchange with the UE in a certain period.

##### 8.1.9.1.2	NRM service for IoT power saving

The following new NRM service listed in table 8.1.b.1.2 are introduced:

Table 8.1.b.1.2-1: List of new NRM services for IoT power saving

| Service Name | Known Consumer(s) | Communication Type |
| --- | --- | --- |
| IoT Power saving service | Application layer entity | Request/response and subscribe/notify |

#### 8.1.9.2	Impacts on architecture, services, entities

This solution has no architecture impacts.

This solution creates new NRM service at NRM for IoT power saving.

#### 8.1.9.3	Solution evaluation

This solution provides a single IoT power saving service to the IoT service provider/developer to achieve the power saving. It aggregates multiple network northbound service and expose a single service API to the ASP which simplifies the interactions between the IoT service provider/developer and the 3GPP network to utilize the related network features.

### 8.1.10	Technical Solution # 10: Single NRM service for network service QoS control for applications

#### 8.1.10.1	Solution description

This solution proposes to address the technical gaps identified in clause 4.1 regarding the support for various application requirements at NRM.

To achieve an API design involving different use cases/application with different QoS characteristics, the proposed solution is to consider a single API and with inputs – defining the basic QoS requirements as highlighted below:

Target QoS requirements:

- Data rate

  - Maximum Uplink Data rate

  - Maximum Downlink Data rate

- Delay Characteristics

  - Maximum Uplink Delay

  - Maximum Downlink Delay

  - Round trip Delay

  - Delay Variance

- Maximum Packet Loss Rate

NOTE: Either Maximum Uplink Delay and Maximum Downlink Delay or Round trip Delay can be provided.

##### 8.1.10.1.1	Procedures

This subclause describes the procedure for QoS control service creation/updating/deletion based on VAL server application QoS requirements.

Figure 8.1.10.1.1-1 illustrates the procedure for the QoS control service.

Figure 8.1.10.1.1-1: QoS control service procedure

- The VAL server triggers SS_ NRM_QoS_Service_Create/Update/Delete request to the NRM server to create/update/delete network resources for one or more users belonging to one or more VAL services based on the application QoS requirements. This includes target QoS requirements (e.g. Data rate, Delay characteristics and Packet loss) for the corresponding VAL UEs or group of VAL UEs or VAL services.

- The NRM server shall authorize the request and evaluates the input required for Nnef_AFSessionWithQoS API trigger.

NOTE:	If the NRM is considered to be trusted by the operator, the NRM can use the Npcf_PolicyAuthorization_Create/Update/Delete request message to interact directly with 5GC (i.e. PCF) to request/update/delete resources.

3.	The NRM server shall trigger Nnef_AFSessionWithQoS_Create/Update/Delete request towards 5GC (via NEF) to request/update/delete resources for the VAL services.

4.	The NRM server shall provide SS_NRM_QoS_Service_Create/Update/Delete response to the VAL server with the result. This will include information either per VAL UE or per set of VAL UEs or VAL services, as indicated by the request of the VAL server in step 1.

##### 8.1.10.1.2	Information flows

###### 8.1.10.1.2.1	SS_NRM_QoS_Service_Create request

Table 8.1.10.1.2.1-1 describes the information flow for QoS service create request from the VAL server to the NRM server.

Table 8.1.10.1.2.1-1: SS_NRM_QoS_Service_Create request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| Application Server address | M | The endpoint address of the Application Server, where the Application traffic flows. |
| List of VAL UE IDs or VAL user IDs | O (see NOTE) | List consisting of one or more VAL UE IDs and/or VAL user IDs for whom the network resource creation is requested. |
| VAL group ID | O (see NOTE) | The VAL group ID for whom the network resource creation is requested. |
| Target QoS requirement | M | The target QoS requirement corresponds to the VAL service QoS requirements as applied for a UE or group of UEs (E.g. Data rate, Delay characteristics, Packet loss) and the corresponding Application traffic flow. |
| NOTE: Either of the information elements should be present. |  |  |

###### 8.1.10.1.2.2	SS_NRM_QoS_Service_Create response

Table 8.1.10.1.2.2-1 describes the information flow for QoS service create response from the NRM server to the VAL server.

Table 8.1.10.1.2.2-1: SS_NRM_QoS_Service_Create response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the creation of the network resource with the underlying network. |

###### 8.1.10.1.2.3	SS_NRM_QoS_Service_Update request

Table 8.1.10.1.2.3-1 describes the information flow for QoS service update request from the VAL server to the NRM server.

Table 8.1.10.1.2.3-1: SS_NRM_QoS_Service_Update request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| Application Server address | M | The endpoint address of the Application Server, where the Application traffic flows. |
| List of VAL UE IDs or VAL user IDs | O (see NOTE) | List consisting of one or more VAL UE IDs and/or VAL user IDs. |
| VAL group ID | O (see NOTE) | The VAL group ID. |
| Target QoS requirement | O | The target QoS requirement corresponds to the VAL service QoS requirements as applied for a UE or group of UEs (E.g. Data rate, Delay characteristics, Packet loss) and the corresponding Application traffic flow. |
| NOTE: Either of the information elements should be present. |  |  |

###### 8.1.10.1.2.4	SS_NRM_QoS_Service_Update response

Table 8.1.10.1.2.4-1 describes the information flow for QoS service update response from the NRM server to the VAL server.

Table 8.1.10.1.2.4-1: SS_NRM_QoS_Service_Update response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the update of network resource with the underlying network. |

###### 8.1.10.1.2.5	SS_NRM_QoS_Service_Delete request

Table 8.1.10.1.2.5-1 describes the information flow for QoS service delete request from the VAL server to the NRM server.

Table 8.1.10.1.2.5-1: SS_NRM_QoS_Service_Delete request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| Application Server address | M | The endpoint address of the Application Server, where the Application traffic flows. |
| List of VAL UE IDs or VAL user IDs | O (see NOTE) | List consisting of one or more VAL UE IDs and/or VAL user IDs. |
| VAL group ID | O (see NOTE) | The VAL group ID. |
| NOTE: Either of the information elements should be present. |  |  |

###### 8.1.10.1.2.6	SS_NRM_QoS_Service_Delete response

Table 8.1.10.1.2.6-1 describes the information flow for QoS service delete response from the NRM server to the VAL server.

Table 8.1.10.1.2.6-1: SS_NRM_QoS_Service_Delete response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the deletion of network resource with the underlying network. |

##### 8.1.10.1.3	NRM service

The proposal is to use a single NRM service for QoS control.

The following table 8.1.1.3-1 provides example of QoS requirements and the corresponding applications:

Table 8.1.1.3-1: List of examples QoS requirements and the corresponding applications

| Max Data rate |  | Delay Characteristics |  |  |  | Max Packet loss rate | Applications |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Uplink | Downlink | Max Uplink Delay | Max Downlink Delay | Round trip Delay | Delay variance |  |  |
|  | 100 Mbps - 1 Gpbs |  |  | 10 ms | 30 ms | 0.01 | Consumption of VR content via tethered VR headset (NOTE 1) |
| 5 Mbps | 100 Kbps | 50 ms | 50 ms |  | 20 ms | 0.001 | IoT (Factory sensors) (NOTE 2) |
| 10 Mbps | 10 Mbps | 20 ms | 20 ms |  | 20 ms | 0.001 | IoT (wearables) (NOTE 2) |
| 600 Mbps | 600 Mbps |  |  | 300 ms | 30 ms | 0.01 | Immersive VR navigational application (NOTE 3) |
| 25 Mbps | 1 Mbps |  |  | 5 ms | 10 ms | 0.001 | V2X application (NOTE 4) |
| NOTE 1: Table 7.6.1-1 in TS 22.261 [17]. NOTE 2: Table 7.7-1 in TS 22.261 [17]. NOTE 3: Table 7.11-1 in TS 22.261 [17]. NOTE 4: Table 5.5-1 in TS 22.186 [18]. |  |  |  |  |  |  |  |

#### 8.1.10.2	Solution evaluation

This solution addresses the technical gap 4-1-2-1 on how to enhance NRM services API for various services and applications, ensuring ease of API usage and at same time ability to support services with commonly used QoS parameters for an API.

The key differentiating aspect of this solution (when compared with other proposed solution addressing the same technical gap), is the usage of single API with minimum parameters that cater to different service/application requirements, thus simplifying the interactions between ASP and the 3GPP network.

If the proposed API in this solution can be used as an independent new API or will be used further to update the existing SS_NetworkResourceAdaptation defined in 3GPP TS 23.434 [3] clause 14.4.2 can be decided during overall evaluation of the technical gap or during the normative phase.

### 8.1.11	Solution #11: New SEAL service for power saving in IoT devices

This solution corresponds to the technical gap described in clause 4.4, where the use of SS_NetworkResourceAdaptation and MonitoringEvent APIs to meet IoT application requirement, e.g., power saving, is not clear.

For the case of IoT devices, the NRM may be configured with DRX and eDRX parameters value that is mapped with different power saving modes which may be requested by the VAL server.

NOTE 1:	It is up to the PLMN operator to determine which DRX parameters the NRM may request to reconfigure the VAL UEs.

#### 8.1.11.1	Solution description

The solution presents two procedures which correspond to two cases:

-	Clause 8.1.11.3 describes the procedure related to the case when VAL server triggers power saving assist service for power saving in IoT devices. In this scenario, the VAL server requires a desired power saving mode for the target devices.

-	Clause 8.1.11.4 describes the procedure related to the case when VAL client triggers power saving assist service for power saving in IoT devices. In this scenario, the VAL client triggers a desired power saving mode, e.g.,, as the VAL UE prepares for running in low battery in the near future, so power saving mode is required.

The related information element is described in clause 8.1.11.2.

#### 8.1.11.2	Information elements

Table 8.1.11.2-1: Power saving assist request

| Information element | Status | Description |
| --- | --- | --- |
| VAL User or VAL UE(s) ID(s) (NOTE) | M | The identity of the VAL user whose UE(s) are target for power savings configuration. Alternatively, is the UE ID on which the NRM client resided |
| VAL Service ID | M | The identity of the VAL service |
| Power saving requirements | M | The desired power saving requirements for the VAL UE(s), e.g., power saving mode (e.g., power-saving, balanced, performance) to be enforced for the target user(s) |
| NOTE: If the consumer is the NRM client, this IE shall be the UE ID. If the consumer is VAL server, the IE may be a list of UE IDs or identity of VAL user. |  |  |

Table 8.1.11.2-2: Power saving configuration request

| Information element | Status | Description |
| --- | --- | --- |
| VAL UE ID | M | Identity of the VAL UE which is a target for power savings configuration |
| Requested power saving configuration | M | The desired power saving configuration to be updated for the target user, e.g., DRX and eDRX configurations in 3GPP TS 38.331 [25] clause 6.3, clause 6.2, respectively. |

Table 8.1.11.2-3: Power saving configuration response

| Information element | Status | Description |
| --- | --- | --- |
| VAL UE ID | M | Identity of the VAL UE which is a target for power savings configuration |
| Result | M | Indicates whether the result of the power saving configuration request is successful or not |
| Accepted power saving configuration | O (NOTE) | Indicates the accepted power saving configuration |
| NOTE: This information element is present if the accepted power saving configurations are different from the desired one. |  |  |

Table 8.1.11.2-4: Power saving assist response

| Information element | Status | Description |
| --- | --- | --- |
| VAL User or VAL UE(s) ID(s) (NOTE) | M | The identity of the VAL user whose UE(s) are target for power savings configuration. Alternatively, is the UE ID on which the NRM client resided. |
| >Result | M | Indicates whether the result of the power saving configuration request is successful or not |
| >Accepted power saving configuration | O | Indicates the accepted power saving configuration |
| NOTE: If the consumer is the NRM client, this IE shall be the UE ID. If the consumer is VAL server, the IE may be a list of UE IDs or identity of VAL user. |  |  |

#### 8.1.11.3	Procedure case 1

Figure 8.1.11.3-1 describes the procedure when the VAL server triggers power saving assist service for power saving in IoT devices.

Figure 8.1.11.3-1: Procedure for VAL server triggers power saving assist service

1.	VAL server requests the NRM Server to assist power saving of the VAL user(s) that are part of VAL Service. The request contains the list of VAL UEs and VAL Users that are targets for the power saving, VAL service ID, power saving requirements, e.g., mode (power-saving, balanced, performance) (see Table 8.1.11.2-1).

2.	NRM server determines, based on the VAL service ID, UE(s) access type and power saving mode, the power saving configuration, network parameters and communication pattern parameters for each requested VAL UE(s). Furthermore, the SEAL Server may determine the VAL UE ID(s) associated with the VAL user(s) included in step 1.

3.	NEM Server interacts with 3GPP CN (e.g., CpProvisioning and/or NpConfiguration APIs) to provision the VAL UE(s) network parameters and communication pattern parameters determined in step 2.

4.	NRM Server interacts with NRM Client with recommended power saving configuration for the UE (e.g., the determined power saving configuration in step 2) (see Table 8.1.11.2.-2).

5.	The target UE interacts with 3GPP network as per clause 4.2.2.2.2 of 3GPP TS 23.502 [11], to indicate the desired power saving configuration, e.g., DRX or eDRX configuration. The UE receives from 3GPP RAN the accepted power saving configuration.

NOTE 2:	It is up to the 3GPP system to determine the accepted, i.e., final DRX configurations to be reconfigured in the VAL UE.

6.	NRM Client responds to the NRM Server with power saving configuration response and indicates the accepted power saving configuration (see Table 8.1.11.2-3).

7.	NRM Server responds to the VAL Server with power saving assist response (see Table 8.1.11.2-4).

#### 8.1.11.4		Procedure case 2

Figure 8.1.11.4-1 describes the procedure when the VAL client triggers power saving assist service for power saving in IoT devices.

Figure 8.1.11.4-1: Procedure for VAL client triggers power saving assist service

0.	VAL Server may interact with VAL UE(s) that are part of VAL Service for power saving request.

1.	NRM Client requests the NRM Server to assist power saving of the VAL UE(s). The NRM client indicates the desired power saving configuration to the NRM Server (see Table 8.1.11.2-1).

2.	Steps 2-6 as in Figure 8.1.11.3-1.

3.	The NRM Server responds to the NRM Client with power saving assist response (see Table 8.1.11.2-4).

4.	 The VAL UE may interact with VAL Server for power saving response.

#### 8.1.11.5	Solution evaluation

The solution addresses the existing technical gap#4 in clause 4.4.2 related to IoT power saving scenarios. The solution describes two scenarios based on the entity that triggers power saving assist service for power saving in the VAL UE: scenario 1 is when the VAL server triggers the required power saving mode. Scenario 2 is when the VAL client triggers the required power saving mode. The solution describes an end-to-end behaviour to achieve power saving for VAL UEs that addresses the gap mentioned earlier.

### 8.1.12	Technical solution 12: Solution for stage 2 services design in SEAL specifications

#### 8.1. 12.1	Solution description

This solution addresses technical gap#7.

The API definition in 3GPP TS 23.434[3], 3GPP TS23.435[4], 3GPP TS23.436[5], 3GPP TS23.437[6], 3GPP TS23.438[7], 3GPP TS23.482[12] should be changes to SEAL services (list API The following way:

- List of SEAL API should be reverted to List of SEAL service

- API description should be reverted to service description

- The procedures should show linkage to the SEAL service.

The following are example about how to modify the TS (taking SS_LocationInfoRetrieval as the example):

*******************************first change****************************************

Table 9.4.1-1: List of SEAL APIs Services for location management

| API Name Service Name | API Service Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| … | … | VAL server | Subscribe/Notify |
| SS_LocationInfoRetrieval | Obtain_Location_Info | VAL server | Request /Response |

*******************************Second change****************************************

9.4.4	SS_LocationInfoRetrieval API service

9.4.4.1	General

API  Service description: This API enables the VAL server to obtain UEs location information from the location management server over LM-S.

9.4.4.2	Obtain_Location_Info operation

Service API operation name: Obtain_Location_Info

Description : Request UEs location information.

Known Consumers: VAL server.

Inputs: Refer subclause 9.3.2.3

Outputs: Refer subclause 9.3.2.2

See subclause 9.3.9 for the details of usage of this API operation.

**************************************Third change************************************

9.3.9	On-demand usage of location information procedure

The VAL server can request UE location information at any time by sending a location information request to the location management server, which may trigger location management server to immediately send the location report.

Figure 9.3.9-1 illustrates the high level procedure of on demand usage of location information. The same procedure can be applied for location management client and other entities that would like to subscribe to location information of VAL user or VAL UE.

Pre-condition:

-	The LM Server has confirmed the target UE is associated with other UEs and they are sharing the same location.

-	The LM Server has obtained the UE’s location who is associated with the target UE.

Figure 9.3.9-1: On-demand usage of location information procedure

1.	VAL server sends a location information request for SS_LocationInfoRetrieval to the location management server to request locations for one or more VAL UEs or VAL user(s) which may include the location QoS that contains the location accuracy, response Time and QoS class as defined in clause 4.1b of TS 23.273 [50].

2.	The location management server checks if there is stored and valid UE location report (e.g. the timer for the stored UE location report is not expired) for the target UE. If available, it reuses the stored UE location information and reports it to the VAL server. If there isn’t, it acquires the latest location of the UEs being requested, by triggering an on-demand location report procedure as described in subclause 9.3.4, or from PLMN operator, and checks if the obtained location of the VAL UE could meet the location QoS requirements (if any).

If the LM Server identified that target UE is associated with other UE(s) and they are sharing the same location as specified in clause of 9.3.23.2 or 9.3.23.3, it determines to reuse the associated UE's location data instead of obtaining the target UE's location data directly. The LM Server may select the associated UE(s) based on the requested LCS QoS, and the operator's policies, etc. But if the LM Server doesn’t store the associated UE's location data or the stored location data is invalid (e.g. the valid timer for the confirmation of sharing location information is expired, or the verify location sharing response is negative), the LM Server will ask the target UE to report the UE location data as described in clause 9.3.4.

3.	Then, location management server immediately sends the location information report for SS_LocationInfoRetrieval including the latest location information of associated UE(s) or acquired of target one or more VAL users or VAL UEs. The location management server may report the location to the VAL server considering the location information received via non-3GPP positioning technologies (e.g. GNSS, Bluetooth), for instance, to improve the location accuracy.

4.	VAL server may further share this location information to a group or to another VAL user or VAL UE.

NOTE:	For other entities, the step 4 can be skipped if not needed.

**************************************The fourth change************************************

9.3.2.3	Location information request for SS_LocationInfoRetrieval

Table 9.3.2.3-1 describes the information flow from the VAL server to the location management server and/or from the location management server to the location management client for requesting an immediate location information report.

Table 9.3.2.3-1: Location information request

| Information element | Status | Description |
| --- | --- | --- |
| Identity list | M | List of VAL users or VAL UEs whose location information is requested |
| VAL service ID | O | Identity of the VAL service for which the location information is requested. |
| Requested location information | O | Identifies what location information is requested |
| Requested location access type (NOTE 4) | O (NOTE 1) | Identifies the location access type for which the location information is requested, e.g. as described in TS 23.273[50] and TS 29.572[51]. |
| Requested positioning method | O (NOTE 1) | Identifies the positioning method for which the location information is requested, e.g. as described in TS 23.273[50] and TS 29.572[51]. If the requested VAL user/UE is not the receiving VAL user/UE hosting the location management client, the UE positioning method includes PC5 SEAL LM, BT, WiFi. |
| Location QoS | O (NOTE 3) | Definition of the location Quality of Service for which the location information is requested (see NOTE 2). |
| Requested velocity indication | O (NOTE 5) | It indicates whether the velocity of the requested VAL users/UEs is needed. |
| List of association IDs | O (NOTE 6) | Identifies list of associations with other UEs may share the location information when they are at the same location. |
| NOTE 1: This e element is only applicable for request sent from the location management server to the location management client. NOTE 2: The definition of location QoS has been defined in clause 4.1b of TS 23.273 [50] and the clause 6.1.6.2.13 of TS 29.572 [51]. NOTE 3: The element is only applicable for the information flow from the VAL server to the location management server. NOTE 4: The non-3GPP access as defined in TS 23.273[50] and TS 29.572[51] is out of scope of the present specification. NOTE 5: The definition of the velocity refers to 3GPP TS 23.271 [60]. NOTE 6: The association IDs are known to the UE based on offline methods and are out of scope of this specification. The requestor can be part of multiple associations and one association can have two or more UEs. |  |  |

#### 8.1.12.5	Solution evaluation

The solution addresses the existing technical gap in clause Gap# 4.7.2-1. It provides an example about how to modify stage 2 SEAL specifications according to stage 2’s scope.

**************************************The end of changes************************************

### 8.1.13	Technical solution#13: Service design guidelines for common functionalities

#### 8.1.13.1	Solution Description

This paper proposes service design guidelines for common functionalities related to technical gap#8. In alignment with SEAL approach for common services for application enablement, this paper recommends that the common requirements and functionalities for different scenarios/applications shall be compiled into common enablers/services that can be reused by any application with the same requirements.

Generic services

The services shall address common requirements which can serve multiple applications. For example, services shall not be designed specific to the applications such as XR, IoT etc duplicating same functionalities but for different applications. The services shall be usable for multiple applications which have similar requirements.

Functionality driven service

The service design shall focus on the functionalities instead of the applications itself. For example, consider applications that require:

- Latency, e.g., Low latency

- Reliability, e.g., High reliability

- Data rate, e.g., High data rate

- Bandwidth

and/or its combinations, can be classified into applications that require QoS requirements (e.g., low latency and high reliability). Hence, instead of designing an extensive list of services for the above-mentioned functionalities or its special combinations to satisfy the application, the service design shall be driven by the common functionalities (e.g., QoS requirements) exposed. This will minimize the design of multiple services for the same functionality causing duplication.

Improvement of existing SEAL services

An important part of service designing is the improvement of existing SEAL services based on the service design guidelines as addressed in technical gap#8. This can be done by evolving the existing SEAL service input parameters to satisfy the new network capabilities (e.g. multi-modality).

The focus shall be on the functionality, e.g., the QoS requirements in these scenarios. For example, the existing services (SS_NetworkResourceAdaptation and SS_NetworkResourceMonitoring) input parameters lacks the support for requesting QoS requirements per traffic type. The proposed service design to improve the input parameters of the NRM services shall make it usable in different new aplications with latest network capabilities instead of designing multiple services with similar functionalities for different applications.

External TR for SEAL guidelines

Produce an external TR with SEAL Guidelines that explain how SEAL enablers can be used, with examples that show how applications can profile the SEAL enablers based on their requirements and needs.

#### 8.1.13.2	Solution evaluation

This solution addresses technical gap#8 and improvement of existing SEAL services based on the guidelines.

## 8.2	Solutions for SEAL services adoption gaps

### 8.2.1	Adoption Solution#1: Service based Architecture of 3GPP system supporting SEAL services

#### 8.2.1.1	Solution description

This solution is to solve GAP# 5-1-2-1.

Figure 8.2.1-1 Service based Architecture of 3GPP system supporting SEAL services

In the figure 8.1.1-1, SEAL services layer represents the SEAL server(s) supporting SEAL services and the SEAL client(s) (if applicable) defined in the SEAL specification 3GPP TS 23.434 [3].

SEAL services which are part of the 3GPP system are deployed either by the MNO or third party.

The interactions between the vertical application server(s) and SEAL server(s) for discovery, authentication and authorization are supported by CAPIF as specified in 3GPP TS 23.222 [9]. The VAL server acts as CAPIF's API invoker and SEAL server acts as CAPIF's API exposing function.

SEAL server(s) invokes service APIs exposed by 3GPP core network functions directly over the 3GPP core network SBA when deployed as trusted AF in MNO’s trusted domain. Alternatively, SEAL server(s) invokes service APIs exposed by 3GPP core network functions via NEF as a non-trusted AF e.g., when deployed by the third party.

SEAL server(s) invokes service APIs exposed by the OAM system over the OAM SBMA when deployed by MNO as specified in 3GPP TS28.533[14].

SEAL clients take two different responsibilities:

1) consumer of SEAL server API(s)

2) providing required terminal side information/resource to support the SEAL server function(s)

SEAL services API are provided and exposed by MNOs or third party, and they are consumed by the Application specific layer (VAL layer) or vertical application enabler layer (e.g., VAE server, UAE server), Edge enabler layer (e.g., EES, ECS), SEAL client(s) or third party’s platform.

NOTE 1:	 It is up to application developer’s implementation how to invoke the SEAL server’s APIs in their application layer software.

NOTE 2:	3The API invokers as defined in 3GPP TS 23.222[9] can either be on the network side (Application server) or on the UE side (e.g., SEAL clients), to use SEAL server API(s).

#### 8.2.1.2	Functional Elements

NOTE:	No new function elements are added to the existing 3GPP system.

#### 8.2.1.3	Reference Points

NOTE:	No new reference points added to the existing 3GPP system.

#### 8.2.1.4	Solution evaluation

This solution provides the holistic picture providing the role and responsibility of SEAL service layer within the complete 3GPP system perspective. It clarifies the SEAL services can be implemented as part of 3GPP system and can well avoid some misleading.

### 8.2.2	Adoption Solution#2: Layered representation of 3GPP network exposure system

#### 8.2.2.1	Solution description

This solution is to solve adoption gap#3.

Network Services

(e.g., AMF, NEF, NWDAF)

O&M Services

(e.g., FCAPS, slicing)

SEAL services servers

(e.g., NRM/SEAL DD for connection over 5/4G, NSCE for 5G Slice, Group, location)

Southbound Interface (SBI)

(Network APIs)

Northbound Interface (NBI)

(Service APIs)

APIs publish

APIs discovery

VAL server

(V2X)

VAL server

(UAS)

VAL server

(IoT)

VAL Client

(V2X, UAS, IoT, …)

SEAL Clients

Application Enablement layer services over 5G/EPS

SEAL-C APIs

UE/Device

5G Network services

Application Layer

UE Modem

APIs

APIs

APIs

APIs

APIs

APIs

CCF

AEF (Authorization checking， message routing)

APIs

APIs

Network connection layer

Service abstraction layer/value added service layer

Exposure framework

Application layer

APIs publish

AT command

Uu

SEAL-UU

Figure8.2.2.1-1: Layered representation of SEAL service layer in 3GPP network exposure system

#### 8.2.2.2	Functional Elements

##### 8.2.2.2.1	3GPP Exposure  layer

3GPP Exposure layer acts entry points for exposing services APIs supported by 3GPP system, to a services APIs customer (the customer can be Application layer/or a third party’s re-exposure system). The APIs exposed by exposure layer contains both service APIs and framework APIs.

the framework APIs and function entities  supports the following functions as defined as CAPIF in 3GPP TS 23.222[9]:

- CCF and framework APIs exposed by CCF over CAPIF-1: Providing Common functions for a system to expose different APIs: including operate related (API discovery, Invoker account creation) for API consumers [e.g., Application owner] making API service subscription and API invoker onboarding.

-	AEF and framework APIs: Providing API invoking control (validating the API invokers and making authorization checking) and managing the messages routing to the  entity supporting services required by APIs, when receiving a service API calling message request from a running application.

The services exposed by services APIs can be from different domain e.g., including what supported by network connection layer (3GPP Core and OAM) and function entities at application enabler layer.

##### 8.2.2.2.2	3GPP service layer

3GPP service layer includes the legacy MNO services like IMS, SMS, and 3GPP application enabler layer services.

3GPP application enabler layer provides value added service based on network layer capabilities (including capabilities supported by Core network and OAM). it provides the following services based on generality level as described in clause 8.2.2.2.2.1, 8.2.2.2.2.2, 8.2.2.2.2.3:

NOTE:	In some organizations, this layer is also called value added service layer or service abstraction layer.

###### 8.2.2.2.2.1	Edge application enabler layer

The services and APIs provided by EEC/EES/ECS as defined in 3GPP TS 23.558[20] at this layer, enable an application on mobile UE to discover and select an edge application server (EAS) deployed at Edge site (at or close to 3GPP network border), and application server rediscovery and reselection due to UE mobility or overloaded.

The services and APIs provided by ECSP management system as defined in3GPP TS 28.538 [19] at this layer, enable an Entity within ECSP or third party to trigger EAS instantiation on Edge nodes.

Each service and API at the layer is generic to any application type. It also applicable to SEAL layer services (SEAL clients to find SEAL service server when deployed at edge).

###### 8.2.2.2.2.2	SEAL service layer

The services at this layer defined in 3GPP TS 23.434[3], 3GPP TS23.435[4], 3GPP TS23.436[5], 3GPP TS23.437[6], 3GPP TS23.438[7], 3GPP TS23.482[12] provide the value-added services to enable application layer to utilize 3GPP network and UE capabilities in an easy-to-use way. SEAL services and APIs are generic to any application type.

SEAL layer includes SEAL services server(s) and SEAL client(s).

- SEAL services server(s): it is implemented in network by MNO or 3rd part and are authorized to (1) provide APIs to application and SEAL client, and (2) invoke network APIs to get 3GPP network services.

- SEAL client(s): it is implemented in UE and can be provided by same organization as SEAL service server or other origination e.g., UE vendor/OS and are authorized to (1) provide APIs to application, (2) invoke APIs from SEAL service server to get Network services, or (3) trigger UE modem to initiate NAS procedures to invoke network capability so as to provide service result to application.

Services created by the combination use of different network connection layer capabilities from one or multiple Network Functions deployed in telecom system, may potentially combining the resources/information of client on the UE.

Services exposed at this layer normally is larger granularity than the API service exposed by single network function at network connection layer and provide an application layer meaningful function. It could simply the application layer implementation compared to directly invoke network connection layer APIs one by one.

###### 8.2.2.2.2.3	Vertical specific services layer

A service at this layer provides certain application layer service functions, which is specific for certain vertical application.

In 3GPP, the application specific service includes 3GPP TS 23.255 [22] for UAV, or 3GPP TS 23.286 [21] for V2X.

##### 8.2.2.2.4 3GPP user communication service layer

It provides another type of service layer services, which mainly refers to the legacy user-orient operator applications, like IMS voice, SMS. These applications can also be exposed as the services to a third-party application.

The APIs specified in 3GPP includes 3GPP TS 23.228 [24], or 3GPP TS 23.239 [23]

##### 8.2.2.2.4	3GPP network connection layer

The smallest granularity services that a 3GPP network layer function entity or OAM function entity, or a 3GPP UE is able to support e.g., 3GPP network connectivity services, QoS control, 3GPP position, slice creation/updating/deletion.

Network functions and capabilities defined at this layer are designed generally to serve multiple applications/scenarios.

NOTE:	The principles of network layer design normally consider how to use one network capability to support multiple scenarios. So normally the capabilities defined at this layer is in a small granularity. These capabilities can be used as element capabilities and been further aggregated to a larger granularity added value service.

In this layer, it also includes the NAS procedure supported by both UE and network layer so as to provide service result to application by UE.

#### 8.2.2.3	Relationship between different layer

APIs from 3GPP network connection layer and APIs from SEAL service server of application enabler layer can be provided by MNO and offered certain services towards application layer via CAPIF.

3GPP service layer consumes the API services from 3GPP network connection layer and also terminal side APIs services.

APIs from SEAL client can be provided by MNO or 3rd party e.g., UE vendor and offer certain SEAL layer services towards application. When these API applies, the SEAL client may further invoke APIs from SEAL service server to get Network services, or SEAL client may request network layer services (e.g., PDU session) via UE modem.

### 8.2.2.2	Solution evaluation

This solution clarifies the role and responsibility of SEAL service layer from the whole 3GPP system perspective, which are missing from 3GPP official document. Such layered representation of whole 3GPP network exposure system in the network exposure language will be helpful for customers to understand SEAL services better.

### 8.2.3	Adoption solution#3: business relationship involving SEAL Client provider

#### 8.2.3.1	Solution description

This solution addresses adoption gap#5.

VAL service provider may get SEAL services by establishing the service agreement with SEAL server service provider directly as figure 8.2.3.1-1, or indirectly based on the service agreement between VAL service provider and SEAL client service provider, and SEAL client service provider and SEAL server service provider as figure 8.2.3.1-2.

As figure 8.2.3.1-1 shows, some SEAL clients are required to provide terminal side information/resource to support some functions of SEAL service serve. Thus, SEAL server services provider may need to implement SEAL clients, to enable the SEAL server services business toward to VAL service provider.

As figure 8.2.3.1-2 shows, SEAL client may support the function of invoking the API from SEAL service server, then re-exposing them to Application Clients as the local API or SDK service. In this case, VAL service provider who own Application Clients needs to setup service agreement with SEAL client service provider and SEAL client provider should also setup service agreement with SEAL server service provider. Some SEAL client may also provide function based on UE model capabilities, which may potential requires the PLMN business agreement between SEAL client provider and PLMN operator.

Figure 8.2.3.1-1: Business relationships for VAL services (SEAL client provided by SEAL server provider)

Figure 8.2.3.1-2: Business relationships for VAL services

The VAL user belongs to a VAL service provider based on a VAL service agreement between the VAL user and the VAL service provider. The VAL service provider can have VAL service agreements with several VAL users. The VAL user can have VAL service agreements with several VAL service providers.

The SEAL server service provider and the home PLMN operator can be part of the same organization, in which case the business relationship between the two is internal to a single organization.

The SEAL client service provider can have service arrangements multiple SEAL server service provider.

The VAL service provider can have SEAL service arrangements with multiple SEAL server service providers and the SEAL server service provider can have PLMN operator service arrangements with multiple home PLMN operators. The SEAL server service provider and the VAL service provider or the home PLMN operator can be part of the same organization, in which case the business relationship between the two is internal to a single organization.

The home PLMN operator can have PLMN operator service arrangements with multiple VAL service providers and the VAL service provider can have PLMN operator service arrangements with multiple home PLMN operators. As part of the PLMN operator service arrangement between the VAL service provider and the home PLMN operator, PLMN subscription arrangements can be provided which allows the VAL UEs to register with home PLMN operator network.

The home PLMN operator can have PLMN roaming agreements with multiple visited PLMN operators and the visited PLMN operator can have PLMN roaming agreements with multiple home PLMN operators.

#### 8.2.3.2	Evaluation

This solution addresses adoption Gap#5. It provides more completed business relationship information which cover all the potential business parties related with SEAL layer.

### 8.2.4	Adoption Solution # 4: SEAL deployment involving SEAL Client(s)

#### 8.2.4.1	Solution description

##### 8.2.4.1.1	General description

This solution addresses adoption gap#6.

3GPP TS 23.434 [3] only specifies the deployment alternatives of SEAL server. This solution provides additionally the description of potential SEAL Client(s) deployment alternatives.

SEAL server service provider may provide SEAL Client(s) as terminal SDK on top of terminal OS. Or SEAL Client(s) may be implemented in UE vendor domain (natively supported by OS or as a terminal SDK running over terminal OS).

##### 8.2.4.1.2	Deployment of SEAL Client(s) in the same provider domain of SEAL server(s)

Figure 8.2.1.2-1 illustrates deployment of the SEAL client(s) in the same domain of SEAL server(s). In this case, SEAL server(s) can be deployed in any of the domain described in clause 8.2.1 or 8.2.2, or 8.2.3 of 3GPP TS 23.434 [3].

Figure 8.2.1.2-1: SEAL client(s) deployed in the same domain of SEAL server

##### 8.2.4.1.3	Deployment of SEAL Client(s) in UE vendor domain

Figure 8.2.1.3-1 and Figure 8.2.1.3-2 illustrates deployment of the SEAL client(s) in UE Vendor domain, which is different with the domain of SEAL server. In this case, SEAL server(s) can be deployed in any of the domain described in clause 8.2.1 or 8.2.2, or 8.2.3 of 3GPP TS 23.434 [3].

Figure 8.2.1.3-1: SEAL client(s) deployed in UE vendor domain (within OS)

Figure 8.2.1.3-2: SEAL client(s) deployed UE vendor domain (outside OS)

#### 8.2.4.2	Evaluation

This solution addresses adoption gap#6. It provides the information about the SEAL client deployment which is currently lacking from 3GPP TS 23.434 [3].

### 8.2.5	Adoption Solution#5: Target application use case of SEAL services

#### 8.2.5.1	Description

##### 8.2.5.1.1	General description

This solution is to address the adoption gap #4. It provides the content about the value and beneficial use case and advantages of SEAL services. –

##### 8.2.5.1.2	Overall introduction of SEAL services

This solution is to address the overall summary of the values and beneficial use case(s) of each SEAL service.

The supporting of the following are the key services that make SEAL layer distinct with other non-3GPP basis developer API Platform:

-	NSCE/NRM/SEAL-DD which are specific to the usage of 5G/4G network capabilities.

-	E-MMTel enabler which is specific the usage of IMS system capabilities.

-	LMS utilize both 3GPP based position and non-3GPP based position.

-	AI/ML enabler could support AI/ML service related to mobile UE, leveraging on the 3GPP network-for-AI capabilities

| Service type | Service description | Benefits to application provides |
| --- | --- | --- |
| NRM | - Provide value added APIs by abstrating the capabilties served by core network layer entities MB-SMF/PCF/NEF APIs/SCEF/BMSC, to enable third party to use 3GPP QoS service in a simple way | Simply the usage of App providers (APP deveolper, APP owner) of 3GPP network capabilities to: - Improve qualitiy of application layer communication by using NRM API to tigger the 3GPP network connection with QoS supporting, compared to application layer communication over best effort 3GPP network connection. |
| SEAL DD | - Provide Caching and Distribution between application content server and mobile UE by using various 3GPP network capabilities designed for different type of application data to optimization purpose. For example reliable transmission mechanism specific to URLLC data. | Futher improve QoE of the applications when using Caching and Distribution service, by supporting the usage of 3GPP network capabilities(including control plane and user plane) for QoS optimization |
| NSCE | - Provide the API services to enable third party to retrive the slice SLA requirement automaticly, order slice product from MNO system, mornitor/change slice SLA, by aggregating the API services from OAM and Core network, SEAL layer services (service KQI information) | Provide the required functions(e.g deriving and recomending slice SLA to let a slice customer e.g., enterprise knows how much resource is required for there services) to enable third party to perform self-management operation on their dedicated slice, when NaaS is provided by operator OAM. |
| AI/ML enabler | - AI/ML AIMLE provides APIs for enabling and managing AI/ML services - It leverages the network capabilities of 5G systems to support secure and efficient connection for mobile application to utilize AI services from MNO, | It could enable an MNO operator to use their AI capabilities to launch the new business to AI applications. A mobile AI application can gain from offload the AI task to another node to promote the model performance or latency performance. |
| LMS | LMS provides location management related functionality and location based services to the 3rd party applications. It utilizes the core network related location capabilities and value add on top of it in-order to provide a enriched location services. | Availability of enriched Location information from core network along with the GPS co-ordinates. provides readily available supplementary location based services such as geofencing, location deviation monitoring etc., apart from reporting and fetching of location information. |
| Group management | Group management service provides the group management related operations considering the needs of multiple vertical applications. Apart from regular CRUD operations it provides the value add services like location based group creation, group regrouping, group member registration etc. | Re-usability : Single group management framework catering to multiple vertical needs easing the development time. |
| Configuration management | Configuration management is to enable ASP to provide configuring data applicable to different vertical applications. | Re-usability : Single configuration management framework catering to multiple vertical needs easing the development time. |
| Digital asset | Provides application enablement to Metaverse application by supporting following functionalities: - Digital asset profile management: which includes creating, updating, retrieving and deleting the digital asset profile - Digital asset discovery Digital asset media management – which includes uploading and downloading the digital asset media. | Capabilities to re-use the digital asset across multiple Metaverse applications (e.g. digital assets server provided by the MNO, which is trusted by different Metaverse applications to manage digital assets). |
| Spatial Anhor enabler and spitial mapping enabler | Offers the platform for managing the spatial anchors and spatial maps for metaverse services. Enables the re-use of spatial anchors and spatial maps across multiple verticals and thus improving the metaverse experience for the end users. | Capability to re-use the spatial anchors and maps across multiple verticals. Provides the insight on the spatial anchor usage to improve the effeciency |

##### 8.2.5.1.3	SEALDD service to target application use case

This solution provide clarification on SEALDD service linked to target application use case, and provide analysis on SEALDD mechanism and CDN mechanism.

###### 8.2.5.1.3.1	Use case

CDN is known as a content distribution network. CDN is a global network server system that works together to provide super-fast delivery of Internet content such as website webpages, HTML pages, JavaScript files, stylesheets, images, and videos. When people try to access a website if using a CDN, then visitors will see a copy of the website that is cached from the main server.

Existing widely deployed CDN server use Over-The-Top mechanism. It assumes network layer provide transparent forwarding of IP packets carrying application data. Normally it treats 3GPP mobile network as the same as other type of connection type.

Figure 8.2.5.1.3-1: Existing CDN data transmission by transparent forwarding

3GPP network specifies several mechanisms for improving the transmission quality of application data (e.g., PDU Set based QoS handling as defined by 3GPP TS 23.501 and 3GPP TS 23.502), which requires application layer entity (e.g., application server) support some enhanced features e.g., using RTP or SRTP with RTP Header Extensions for PDU Set Marking as defined in TS 26.522 [179] and together with RTP Payload Format (e.g., H.264 [187] or H.265 [188]);

SEALDD provides some enhanced content delivery functions, which fully use of 5G network to perform content delivery e.g., by providing application layer information required for 3GPP network layer transmission quality enhancement compared to transparent forwarding, thus the SEALDD data can get the best performance supporting from network, can provide content distribution with QoS guarantee which provide better service performance to third party.

###### 8.2.5.1.4.2	Advantages and values of SEALDD Services

As it shows in figure 8.2.5.1.3-2, the application data information will be provided by SEAL DD in the way 3GPP system can read. So that 3GPP network layer can apply some enhancement transmission on SEALDD data.

Figure 8.2.5.1.3-1: SEALDD data transmission using differentiating-processing by 3GPP network

In addition to the user plane co-operation between SEALDD and Network layer, SEAL DD server also use the network APIs from 5G core control plane NF, to trigger PDU Set based QoS handling for different data type, retrieving the network layer information to improving application layer data transmission mechanism (as figure 8.2.5.1.3-3).

Figure 8.2.5.1.3-1: SEALDD utilize 3GPP network APIs for different data

Compared to existing CDN, which provide the same transmission service to different applications. SEALDD could provide different mechanism to meet with different needs of applications as the following:

a) Application layer signalling between AC and AS for different applications with different data transmission requirements.

a.1) Best effort demanding applications

a.2) Low latency demanding applications like signalling for MCX call

- Transmitting the application layer data for different applications with different data transmission requirement:

b.1) Low latency and high reliable demanding application like remote robot/machine control.

b.2) High data rate with low latency demanding application like Realtime XR video applications, Machine vision application….

b.3) Low latency like video call/voice call.

b.4) Low cost demanding like background data

b.5) Low power demanding like IoT device applications.

b.6) Multiple flows synchronisation transmission service like multi-modal XR applications

Benefits to application providers (application owners/developers):

-	 Different applications can get different QoS assurance services for different data type when compared to using over-the-top CDN.

##### 8.2.5.1.4	NRM services to target application use case

###### 8.2.5.1.4.1	Use case

Below list specifies the needs of the various applications utilizing the 3GPP network capabilities :

-	URLLC as mentioned in clause 7.2 of 3GPP TS 22.261

-	High data rate and low latency as mentioned in clause 7.6 of 3GPP TS 22.261

-	Low latency e.g IMS/MCX Voice call/video call

-	Background data transfer

-	Low power consumption

The QoS mechanism supported by 5G network layer (NEF/SCEF or PCF/PCRF, MB-SMF) can be used for the case where the application providers negotiate the QoS parameters required for their applications and sign on business by offline way, or single QoS service for allow all application types can dynamically activating the network QoS service.

Since QoS assurance for different type of target applications requires different network resources and may cause different charging policy with different price, operator may need to further spliiting QoS services into different types of  QoS service. NRM service is defined to transform the 4G/5G network layer APIs to SEAL layer service API, to ease the usage of 3GPP QoS service (e.g., selecting the QoS services based on application needs).

###### 8.2.5.1.4.2	Advantages and values of NRM Services

Figure 8.2.5.1.4.2-1 Capabilities of 5G system transformed by NRM service

The NRM service transforms 3GPP network capabilities and UE capabilities into the following types of service, enabling third party application providers to select and use network layer connection with different QoS services based on application needs, as shown in Figure 8.2.5.1.4.2-1:

1）4G/5G unicast network connection with different QoS

2）4G/5G multicast/broadcast network connection with QoS

3）5G network connection for TSC/TSN data flows

4）5G connection for background data transfer

Editor's Note:	Rel-20 may introduce new APIs to further simplify the use of 5G network capabilities and whether updates to this clause required or not is FFS.

Benefits to application providers (application owners/developers):

-	Simplifies the usage of 3GPP network capabilities by App providers (APP deveolper, APP owner)

-	By using the NRM service APIs, QoE of the application can be improved when compared to application layer communication over best effort 3GPP network connection.

##### 8.2.5.1.5	NSCE to target application use case

###### 8.2.5.1.5.1	Use case

Some of the vertical industry players may rent dedicate slice(s) for their vertical applications. They may intent to have some slice self-management operation by themselves, including determining by themselves the services required slice SLA, ordering a slice product with the determined SLA from MNO, triggering the activation and de-activation, even change the configuration of the slice.

Before vertical industry players determine to utilize dedicated Slice service from MNO, they do not know exactly how much resource is needed to request MNO to configure the dedicated slice. Based on dynamic demand changes at the vertical services, there is a need for vertical industry players to detect and know exactly whether or not to ask MNO to increase/ decrease the resource allocated for the slice initially provisioned.

The NSCE support the required capabilities and services to assist the slice customer to do the above operations.

##### 8.2.5.1.5.2	Advantages and values of NSCE

To enable the slice self-management operation by vertical industry players, NSCE needs to use OAM APIs associated with slices. and core network slice APIs, UE client APIs to produce the NSCE services as Figure 8.2.5.1.5-1. When the NSCE is deployed within operator’s trusted domain, it can access the 5GC directly, otherwise it should access the 5GC via NEF.The following are the example how NSCE can use different services from 3GPP system:

-	Deriving the appropriate recommendation on the Slice SLA parameters, by combining the UE reported application QoE, Slice information from Core network and OAM.

-	Allow third party to monitor and control the slice lifecycle (slice creation/SLA configuration & adaption).

-	Allow third party make the remapping of a slice to the applications

Figure 8.2.5.1.5-1 The required underlying APIs services for NSCE

Benefits to third party:

-	The third party can have a self-manage the dedicate slice (know exactly how much SLA resource are required to serve the applications and sign on an appropriate slice SLA with MNO, make dynamic adaption on slice configuration by monitoring the Slice resource utilization and QoE, Re-mapping of slice to the application).

##### 8.2.5.1.6	Spatial anchors

A spatial anchor is an association between space and service information. The Spatial anchors SEAL service offers spatial anchors related capabilities (i.e., CRUDS) to one or more vertical applications. Spatial anchors enable the authorized users to place digital information at specific locations.

Spatial anchors can be used in many ways:

-	To show the path to the destination without obstructing the user's perception of their proximity (as described in clause 5.1 of 3GPP TR 22.856 [16])

-	To show the content that may be relevant to the user (i.e., potential shopper) (as described in clause 5.1 of 3GPP TR 22.856 [16])

-	To provide personalized message (like the food of a restaurant liked by the user) (as described in clause 5.1 of 3GPP TR 22.856 [16])

Figure 8.2.5.1.6 -1: Localized Mobile Metaverse Services offering relevant information (from clause 5.1 of 3GPP TR 22.856 [r22856])

-	To display extensive information about the products of a shop (as described in clause 5.4 of 3GPP TR 22.856 [16])

Detailed use case it captured in clause 5.1 and clause 5.5 of 3GPP TR 22.856 [16].

###### 8.2.5.1.6.1	Spatial map

The Spatial map SEAL service offers spatial mapping and localization capabilities.

-	Spatial mapping is constructing or updating a map of an unknown location.

-	Localization is tracking an object to identify its location and orientation over time.

For spatial mapping service, sensing data gathered transparently is processed in order to identify the static and transient forms.

Spatial map service can be used in many ways (as described in clause 5.5 of 3GPP TR 22.856 [r22856]):

-	A government conducting a digital city project can build a 3D spatial map of outdoor environment of an entire city or public spaces such as outdoor parks or indoor offices of their government building.

-	A navigation service provider (or Spatial Localization Service) can build a 3D spatial map of outdoor environment of entire roads or public spaces. An operator’s partner or operator also can build a 3D spatial map of outdoor environment.

-	A customer wanting mapping of their indoor environment, e.g., the interior of a commercial space such as the cheese shop as described in 5.4.

Detailed use case it captured in clause 5.5 of 3GPP TR 22.856 [16].

###### 8.2.5.1.6.2	Digital asset

The SEAL digital assets service enables management and usage of digital assets in a secure and controllable way. Examples of digital assets include digital representation (avatar), software licenses, gift certificates, tokens and files (e.g., music files) that have been purchased.

Digital asset service can be used in many ways:

-	Avatar of a human can be used in collaborative and concurrent engineering in an XR meeting (clause 5.3 of 3GPP TR 22.856 [16])

-	To play immersive games and to attend digital live concert (clause 5.6 of 3GPP TR 22.856 [16])

-	To predict remote user’s action (clause 5.9 of 3GPP TR 22.856 [16])

-	To operate patient in critical heath care (clause 5.10 of 3GPP TR 22.856 [16])

-	To have immersive calling experience (clause 5.11 of 3GPP TR 22.856 [16])

Editor's Note:	More changes on usage aspects are FFS.

#### 8.2.5.2	Solution Evaluation

This solution provides the overall summary of functions and values of each SEAL service. It also provides detailed description about the target use case and advantages of several SEAL services. It will be helpful for both system providers and customers to know more about SEAL services.

### 8.2.6	Adoption Solution #6: Alternative solutions for the SEAL mapping to an external SDO‘s system

#### 8.2.6.1	Description

##### 8.2.6.1.1	General description

This solution is to address the adoption gap#5-7-2-1. It provides different alternatives about how to map the SEAL layer services to external API platform e.g., GSMA Open gateway, and how external SDO’s application can use SEAL.

##### 8.2.6.1.2	Alternative 1: SEAL layer services as the SBI of external platform

1)	SEAL layer service as the SBI capabilities.

Figure 8.2.6.1.1-1 SEAL layer APIs are selected by external SDO as SBI

In this solution, it is assumed external platform selects the3GPP API(s) as southbound. Based on their customer’s requirement, it may select to the network layer services by adopting network API. Or it may select enabler layer services by adopting enabler API(s).

External platform may perform API transformation upon 3GPP API(s), if 3GPP API(s) cannot be directly matched to their application layer’s requirement.

##### 8.2.6.1.2	Alternative 2: Standard alternatives for market

##### Figure 8.2.6.1.2-2 Standard alternatives of exposure platform for market

External SDO platform only consume network layer services and defined their enabler layer services and Non-3GPP NBI for third party.

The non-3GPP NBI may expose the same services or different services with what are exposed by 3GPP NBI (possible with different stage-3 API design). It up to each external platform provider/SDO’s decision whether and how to make alignment with 3GPP NBI for the same services.

MNO operator may deploy 3GPP platform (together with 3GPP enabler layer services) and /or external SDO platform.

##### 8.2.6.1.2	Solution evaluation

SEAL layer services provides 3GPP enabler layer services, which are produced based on 3GPP system’s capabilities. 3GPP SEAL layer services can be directly exposed to any applications, for example vertical applications of 5GAA,5GACIA.

From 3GPP perspective, 3GPP API consumers can also be a third-party platform. There are no general restrictions on the API(s) consumer. It could be possible that SEAL layer services are selected by another API platform as SBI, and making API formate transforming when needed to serve their target consumers.It is still up to external SDO’s decision whether to adopt 3GPP SEAL service  or re-specify their own value-add services by extral standard efforts.

# 9	Overall evaluation

## 9.1	Solution evaluations for technical gaps

### 9.1.1	Overall evaluation on technical gap #1

Technical gap #1 focuses on NRM service to support the latest network capabilities, such as multi-modal traffic. Technical solutions 6, 7, and 10 in clause 8.1.6, 8.1.7, and 8.1.10 respectively address this gap.

Solution 6 focuses on application layer requirements by introducing a new set of NRM APIs to support different applications. The NRM services focuses on specific application requirements such as QoS control service for network transport with high data rate and low latency, QoS control service for network transport with low latency, QoS control service for network transport with low latency and high reliability, QoS control service for network transport with low latency & high bandwidth & high reliability. This enables to create, update and delete the QoS control service for network transport with application specific QoS requirements.

On the other hand, solution 7 focuses on the enhancement of input parameters of the network resource adaptation request from VAL server to the NRM server. The network resource adaptation request is enhanced by adding traffic type to indicate different types of traffic involved (for e.g., audio, video, haptic), priority of the traffic types, and QoS requirement per traffic. The enhanced NRM network resource adaptation service allows the application client/server to add the QoS requirements for each traffic type separately. This addition supports newer network capabilities such as multi-modal application traffic that requires to send traffic type specific QoS requirements to the 5GC. This is later mapped with the AFSessionWithQoS for multi-modal service. Hence, it introduces an enhanced NRM service that makes it suitable for scenarios with single or multiple traffic types, irrespective of the application type.

Furthermore, Solution 10 focuses on single NRM service for network service QoS control for applications. The single NRM API includes inputs to define the basic QoS requirements for multiple applications. The input includes QoS requirements such as Data rate (Maximum uplink data rate and Maximum downlink data rate), Delay characteristics (maximum uplink delay, Maximum downlink delay, round trip delay, delay variance), and Maximum packet loss rate.

### 9.1.2	Overall evaluation on technical gap #2

Technical Solution #1 is the only one solution for technical gap #2.

The following texts described in gap#2:” The functions supported by services need to be further simplified and clarified. That is, it is recommended that the service exposed by an API should be Interpretable-value-to- application (e.g., no/less interruption of Video watching, or higher bandwidth assurance for HD video). In this way, it helps third party to clearly know the exact benefits of SEALDD services. This makes it easier for 3rd parties to use the services.”,

This solution provides new SEALDD services based on the application requirements, to make third party know the exact value to their applications.

### 9.1.3	Overall evaluation on technical gap #3

Technical gap #3 contains Gap#4-3-2-1 and Gap#4-3-2-2 as the following:”

Gap#4-3-2-1:	Identify the procedures from the SEAL specification to provide the direct interaction (not via NEF/SCEF) with CN NFs (e.g., PCF, SMF) and/or indirect interaction (via NEF/SCEF) per different deployments and requirements and make changes if required.

Gap#4-3-2-2: Study how to enable GMS to use 5G VN group management services when deployed within trusted”

During the study, it was identified that there is in-consistent description about how location management service interacts with CN NFs, and no description about group management service for direct interaction with CN in 3GPP TS 23.434 [3],

Technical solution 4 provides the solutions to show how change 3GPP TS 23.434 [3] to resolve the in-consistent description about location management service.

However, there are no solution for group management service, due to the need of SA2 feedback on the mechanism of VN group service invoked by trusted AF.  It is suggested to close the study for this gap without waiting for SA2.

### 9.1.4	Overall evaluation on technical gap #4

Technical gap #4 focuses on NRM services supporting IoT devices for dedicated scenarios such as power saving. Both solution 9 in clause 8.1.9 and solution 11 in clause 8.1.11 address this gap.

Solution 9 focuses on the interaction between the NRM server and the 3GPP network to address the gap under consideration. The VAL server requests the power saving service by identifying the requested IoT power saving service (e.g., UE identity, external group identity and application and network pattern parameters) to the NRM server. As a result, the NRM server may invoke one or multiple network related APIs; namely, MonitoringEvent, CpProvisioning and NpProvisioning APIs. Hence, it introduces a single service which may aggregate multiple APIs towards the 3GPP network to achieve the requested power saving service indicated by the VAL consumer.

On the other hand, Solution 11 approaches the gap under consideration from an end-to-end perspective and potential implementation/ deployment considerations. Furthermore, it considers addressing the gap via two cases: Case 1 focuses on the scenario when the VAL server triggers the power saving assist service by identifying a desired power saving mode, whereas Case 2 describes the scenario when the VAL client triggers a desired power saving mode.

Whether the VAL server or the VAL UE is the entity to request the desired power saving mode, the NRM server determines the corresponding power saving configuration, network and communication patterns parameters for the target UE(s) and invokes CpProvisioning and NpProvisioning APIs towards the 3GPP network. As a result, the target UE interacts with the 3GPP network according to the existing procedures indicating the desired power saving configuration (i.e., DRX or eDRX configuration). Hence, this solution not only focuses on the interaction between the VAL consumer and the NRM server, but it rather describes an end-to-end behaviour to achieve the power saving service for IoT devices for all possible scenarios: when the VAL server initiates the power saving service, or when the VAL UE initiates the power saving service.

### 9.1.5	Overall evaluation on technical gap #5

Technical gap #5 identifies the issue on Security Credential in the application enablement specifications. Solution #5 in clause 8.1.5 addresses this gap by providing a method to align SEAL information flows with the security mechanisms specified in 3GPP TS 33.434 [13] and clean application enablement specifications, to avoid using undefined parameters in application enablement specifications.

### 9.1.6	Overall evaluation on technical gap #6

Technical solution#8 is the only solution for technical gap#6. It provides the solution to allow more API consumers, and aligned with existing application development domain implementation method.

### 9.1.7	Overall evaluation on technical gap #7

Technical solution#12 is the only solution for technical gap#7. It provides the solution about how to change the stage 2 TS to align with the stage 2 description style for SEAL services.

### 9.1.8	Overall evaluation on technical gap #8

Technical solution#2 and #3 and #13 are 3 solutions for the gap#8. Solution#2 and Solution#3 provide guidelines for API and SEAL service design from different aspects, and are complementary to each other as the following:

- Technical Solution#2 is about the criterial on what kind of SEAL services and APIs are easy-to-use.

- Technical Solution#3 is more about the working method suggestion on how to design SEAL services to make them easy-to-use.

-Technical solution#13 describes the service and API design based on common requirements from different applications. Technical solution#2 is also about design services based on application requirements. The different between the two solution is how granularity of common requirements from different applications for QoS, Solution#13 cover the case single QoS services for different applications. Solution#2 proposes a smaller granularity level (different QoS type level) for different application needs.

Technical solution#13 also tells the working method like producing the external TR.

## 9.2	Gap evaluation for adoption gaps

### 9.2.1	Overall evaluation on adoption gap#1

The adoption solution#1 is the only solutions for the adoption gap#1.

The following gap is mentioned in adoption gap#1:

- The representation of SEAL services layer within 3GPP system needs to be added.

The adoption solution#1 provides the architectures which includes SEAL services and other part of 3GPP system. Which can be used to show how SEAL services works along with other part of 3GPP system.

### 9.2.2	Overall evaluation on adoption gap#2

The adoption solution#2 is the only solution for the adoption gap#2. It provides the solution to show where SEAL layer locates in the whole layered 3GPP exposure architecture. It is the necessary information to align the understanding within and outside 3GPP.

### 9.2.3	Overall evaluation on adoption gap#3

The adoption solution#5 is the only solution for the adoption gap#3.

It provides the summary of SEAL services and benefits of each.  It also describes target use case, advantages of several SEAL services. Therefore, the solution can be used to tell how SEAL services benefit application developers.

### 9.2.5	Overall evaluation on adoption gap#5

The adoption solution#3 is the only solution for the adoption gap#5. It provides the solution about how to modify the existing SEAL specification, to show the complete business relationship involving SEAL client providers.

Therefore, adoption solution#3 should be selected as the baseline solution for adoption gap#5.

### 9.2.6	Overall evaluation on adoption gap#6

The adoption solution #4 is the only solution for the adoption gap#6. It provides the solution about how to modify the existing SEAL specification, to show how to deploy SEAL clients.

Therefore, adoption solution#4 should be selected as the baseline solution for adoption gap#6.

### 9.2.7	Overall evaluation on adoption gap#7

The adoption solution# 6 is the only solution for the adoption gap#7. It provides the two alternatives for the potential relationship between SEAL services and external SDOs. It is the advice from 3GPP perspectives, and leaves the flexible of external SDO to determine whether to use SEAL services and how to use SEAL services. Therefore, the two alternatives can be put into external TR without precluding any of them.

Therefore, adoption solution# 6 should be selected as the baseline solution for adoption gap#7.

# 10	Conclusions

## 10.1	General conclusions

Editor's note:	This clause will provide general conclusions for the study.

## 10.2	Conclusions of SEAL services technical gap #x

This clause provides the final conclusion for each technical gap.

Technical Gap#1:

Editor’s Note: The conclusion for the technical gap#1 is FFS.

Technical Gap#2: Technical solution#1 is recommended as basis for normative work.

Technical Gap#3: Technical solution#4 is recommended as basis for normative work.

Technical Gap#4:

Editor’s Note: The conclusion for the technical gap#4 is FFS.

Technical Gap#5: Technical solution#5 is recommended as basis for normative work.

Technical Gap#6: Technical solution#8 is recommended as basis for normative work.

Technical Gap#7: Technical solution#12 is recommended as basis for normative work.

Technical Gap#8:

Editor’s Note: The conclusion for the technical gap#8 is FFS.

## 10.3	Conclusions of SEAL services adoption gaps

This clause provides the final conclusion for each adoption gap.

Adoption Gap#1: Adoption solution#1 is recommended as basis for normative work.

Adoption Gap#2: Adoption solution#2 is recommended as basis for normative work.

Adoption Gap#3: Adoption solution#5 is recommended as basis for normative work

Adoption Gap#5: Adoption solution#3 is recommended as basis for normative work

Adoption Gap#6: Adoption solution#4 is recommended as basis for normative work

Adoption Gap#7: Adoption solution#6 is recommended as basis for normative work

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-10 | SA6#63 | S6-244531 |  |  |  | TS skeleton | 0.0.0 |
| 2024-10 | SA6#63 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-244217, S6-244535, S6-244541, S6-244653, S6-244657, S6-244658, S6-244659, S6-244660, S6-244675, S6-244702. | 0.1.0 |
| 2024-12 | SA6#64 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-245606S6-245608, S6-245702, S6-245708,S6-245709, S6-245723, S6-245730, S6-245740, S6-245747 | 0.2.0 |
| 2025-03 | SA6#65 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-250485, S6-250496, S6-250508, S6-250522, S6-250525, S6-250546, S6-250564, S6-250566, S6-250567 | 0.3.0 |
| 2025-04 | SA6#66 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-251214, S6-251304, S6-251306, S6-251309, S6-251310, S6-251314, S6-251429, S6-251430, S6-251431, S6-251432, S6-251434, S6-251458, S6-251479, S6-251493 | 0.4.0 |
| 2025-05 | SA6#67 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-251081, S6-252179, S6-252281, S6-252285, S6-252292, S6-252293, S6-252295, S6-252467, S6-252468, S6-252469. S6-252470, S6-252471, S6-252472 | 0.5.0 |
| 2025-06 | SA#108 | SP-250589 |  |  |  | Presentation for information at SA#108 | 1.0.0 |
| 2025-08 | SA6#68 |  |  |  |  | Implementation of the following pCRs approved by SA6：S6-253101 S6-253102, S6-253303, S6-253308, S6-253393, S6-253394, S6-253395, S6-253396, S6-253398, S6-253399, S6-253603, S6-253604, S6-253683, S6-253684, S6-253685, S6-253688, S6-253706, S6-253743, S6-253744, S6-253762, S6-253763, S6-253764, S6-253765, S6-253766, S6-253767, S6-253780, S6-253781, S6-253782, S6-253783, S6-253784, S6-253786, S6-253790, S6-253796 | 1.1.0 |