| 3GPP TS 26.113 V0.6.0 (2023-05) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Enabler for Immersive Real-time Communication (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	System description	9

4.1	High-level architecture	9

4.2	iRTC client in terminal	9

4.3	Web real-time communication	10

5	Functional components	10

5.1	General	10

5.2	Audio	11

5.2.1	Microphone	11

5.2.2	Pre/post-processor	11

5.2.3	Codec	12

5.3	Video	12

5.3.1	Camera	12

5.3.2	Pre/post-processor	12

5.3.3	Codec	12

5.4	Sensor	12

5.3.1	Measure	12

5.5	Transport protocols	12

6	Session management	13

6.1	General	13

6.2	WebRTC signaling protocol	13

6.2.1	General	13

6.2.2	Protocol and version identification	13

6.2.3	WebSocket URI structure	13

6.2.4	SWAP	14

6.2.4.1	Protocol and version identification	14

6.2.4.2	Transport	14

6.2.4.3	State machine	14

6.2.4.4	Message syntax and semantics	16

6.2.4.4.1	Common message fields	16

6.2.4.4.1.1	Source (source)	16

6.2.4.4.1.2	Message Identifier (messge_id)	16

6.2.4.4.1.3	Message Type (message_type)	17

6.2.4.4.2	Register message	17

6.2.4.4.2.1	 Description	17

6.2.4.4.2.2	Parameters	17

6.2.4.4.3	Response message	18

6.2.4.4.3.1	  Description	18

6.2.4.4.3.2	Parameters	18

6.2.4.4.4	Connect message	18

6.2.4.4.4.1	 Description	18

6.2.4.4.4.2	Parameters	18

6.2.4.4.5	Accept message	18

6.2.4.4.5.1	 Description	18

6.2.4.4.5.2	Parameters	18

6.2.4.4.6	Update message	18

6.2.4.4.6.1	 Description	18

6.2.4.4.6.2	Parameters	19

6.2.4.4.7	Reject message	19

6.2.4.4.7.1	 Description	19

6.2.4.4.7.2	Parameters	19

6.2.4.4.8	Close message	19

6.2.4.4.8.1	 Description	19

6.2.4.4.8.2	Parameters	19

6.2.4.4.9	Application message	19

6.2.4.4.9.1	 Description	19

6.2.4.4.9.2 	Parameters	19

6.2.4.5	 Integrity and security	19

6.2.4.6	 JSON schema	20

6.2.4.7	Protocol operation	21

7	Inter-working	22

7.1	General	22

8	Packet-loss handling	22

8.1	General	22

9	Implementor's guide	22

9.1	General	22

Annex A (informative):                                                                                                                                                                                                                                                 	23

A.1	General	23

Annex B (informative):	24

B.1	General	24

Annex <X> (informative): Change history	25

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

The immersive Real-Time Communication (iRTC) supports a set of features that enable a wide variety of immersive real-time media applications. For capturing media signals in more dimensions than 2D video or mono audio, outputs from multiple cameras and microphones, and the sensors are described. iRTC uses WebRTC with a modular protocol stack as transport, which is integrated into 5G systems, such that applications in need of QoS or other support can receive the necessary services from the network. 3GPP or other SDO’s specifications are referred when necessary.

# 1	Scope

The present document …

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x2]	W3C Recommendation: "WebRTC 1.0: Real-Time Communication Between Browsers".

[x3]	IETF RFC 7478 (2015): "Web Real-Time Communication Use Cases and Requirements".

[x4]	3GPP TS 26.119: "Media Capabilities for Augmented Reality"

[x5]	3GPP TS 26.506: "5G Real-time Media Communication Architecture (Stage 2)"

[x6]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[x7]	3GPP TS 26.114: " IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[x8]	IETF RFC 8825 (2021): "Overview: Real-Time Protocols for Browser-Based Applications".

[x9]	IETF RFC 8835 (2021): "Transports for WebRTC".

[x10]	ITU-R Recommendation BT.601-7 (03/2011): "Studio encoding parameters of digital television for standard 4:3 and wide screen 16:9 aspect ratios".

[x11]	Microsoft: "Microphone Array Geometry Descriptor Format".

[x12]	Apple: "Getting Raw Accelerometer Events".

[x13]	Google: "Sensor Coordinate System".

[x14]	3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification".

[x15]	3GPP TS 37.324: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Service Data Adaptation Protocol (SDAP) specification".

[x16]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[x17]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[x18]	3GPP TS 23.003: "Numbering, addressing and identification".

[x19]	IETF RFC 8829 (2021): "JavaScript Session Establishment Protocol (JSEP)".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

3DoF	Three Degrees of Freedom

6DoF	Six Degrees of Freedom

API	Application Programming Interface

AR	Augmented Reality

DRB	Data Radio Bearer

DTLS	Datagram Transport Layer Security

FFS	For Further Study

FoV	Field of View

HMD	Head-Mounted Display

HTTP	Hyper-Text Transfer Protocol

ICE`	Interactive Connectivity Establishment

IMU	Inertial Measurement Unit

iRTC	Immersive Real-Time Communication

LIDAR	Light Detection and Ranging

MR	Mixed Reality

MNO	Mobile Network Operator

NAT	Network Address Translation

OTT	Over-The-Top

RGB	Red-Green-Blue colour space

RGBD	Red-Green-Blue-Depth

RTC	Real-Time Communication

RTP	Real-time Transport Protocol

SCTP	Stream Control Transmission Protocol

SDO	Standards Developing Organization

SLAM	Simultaneous Localization And Mapping

SRTCP	Secure Real-time Transport Control Protocol

SRTP	Secure Real-time Transport Protocol

SSE	Server-Sent Events

STUN	Session Traversal Utilities for NAT

TLS	Transport Layer Security

ToF	Time of Flight

TURN	Traversal Using Relays around NAT

WebRTC		Web Real-Time Communication

XHR	XMLHttpRequest

XR		Extended Reality

# 4	System description

## 4.1	High-level architecture

The immersive Real-Time Communication (iRTC) system is designed based on the 5G-RTC General Architecture specified in [x5]. Figure 4.1.x illustrates the high-level view of the iRTC system that uses 5G-RTC AF and AS for realizing the services. 5G-RTC AF and AS provide the Control Plane (C-Plane) functionalities for setting up and controlling media and data sessions (U-Plane). The functionalities depend on supported scenarios of collaboration, which are described in [x5]. iRTC system shall support at least one scenario.

Figure 4.1.x: High-level architecture showing two iRTC clients in terminals.

NOTE 1:	Only 5G-RTC AS exists in the media/data path.

NOTE 2:	5G-RTC AF and AS are provided by MNO or 3rd party, depending on adopted collaboration scenarios.

NOTE 3:	Operator B is depicted for collaboration scenario 4. In other collaboration scenarios, "Operator B" is replaced with "Operator A", and the boxes representing the same functionality can be provided by an operator.

## 4.2	iRTC client in terminal

The functional components of a terminal including an iRTC client using 3GPP access are shown in figure 4.2.x. The scope of the present document is to specify the handling and interaction of media and data, which includes their capture and generation, pre/post-processing, and compression. Transport of media and data consists of the encapsulation of the coded media and data in one or more transport protocols, which is shown in figure 4.2.x as the "packet-based network interface" and is illustrated in more detail in the user-plane and control-plane protocol stacks shown in figure 5.5.x.

Figure 4.2.x: Functional components of a terminal including an iRTC client using 3GPP access

The 3GPP Layer 2 protocol to be interfaced with iRTC client is PDCP [x14] for EPC and SDAP [x15] for 5GC, which is used on top of PDCP as shown in clause 4.4.1 of [x16]. It is assumed that the SDAP would be configured without header for both directions in the typical iRTC cases, effectively interfacing with PDCP, as SDAP header would be needed only when more than one QoS flows are multiplexed in a DRB or reflective mapping is enabled. An architecture for XR baseline client can be found in [x4].

NOTE 1:	Functional components in the grey box, except audio and video pre/post-processors, are within the scope of present document, which also specifies output formats of sensor, microphone, and camera.

NOTE 2:	In certain codec types, e.g., avatars, at least the decoders need to be personalized with the information of those who will participate in the communication before or during session setup.

NOTE 3:	Device information is assumed to be stored in the UE and loaded to the iRTC client during session setup.

NOTE 4:	The iRTC client may exchange media and data with external devices tethered over wired links such as USB-C, 3GPP PC5 [x17], or non-3GPP radio access technologies such as Wi-Fi or Bluetooth.

NOTE 5:	Text can be entered via user interface, typically available on display.

## 4.3	Web real-time communication

The iRTC client supports a subset of WebRTC, which enables real-time communication via application programming interfaces (APIs), supporting audio, video, and generic data to be sent between peers [x2]. Functionalities of WebRTC are available as JavaScript APIs for browsers, and libraries for applications [x8]. Information on use cases and requirements of WebRTC can be found in [x3].

# 5	Functional components

## 5.1	General

An iRTC client in terminal requires the following functional components for supporting immersive real-time media including 3D video and spatial audio as well as conventional real-time media including 2D video and mono audio.

## 5.2	Audio

### 5.2.1	Microphone

An iRTC client in terminal can be connected to one or more microphones. The outputs of microphones are audio samples in 16-bit uniform Pulse Code Modulation (PCM) format. An iRTC client or audio infra may identify the direction of each microphone with a coordinate system described in figure A.1.x and table A.1.y.

Figure A.1.x: Microphone array coordinate system

MicrophoneType, whose default value of 0 indicates an omni-directional microphone, identifies the microphones when other types are used. How to assign a value to each microphone type is left to the discretion of the implementation.

| Parameter | Unit | Definition | Note |
| --- | --- | --- | --- |
| Yaw (𝛹) | int | Direction angle | -31416 < 𝛹 ≦ 31416 |
| Pitch (𝛳) | int | Elevation angle | -31416 < 𝛳 ≦ 31416 |
| Roll (𝛷) | int | Rotation angle | -31416 < 𝛷 ≦ 31416 |
| MicrophoneType | int | A number that uniquely identifies microphone type | May be used for indicating vendor-defined microphone types |

Table A.1.y: Microphone description parameters

NOTE 1:	The coordinate system and two angles, yaw and pitch, are originally defined in [X11] for computers.

NOTE 2:	The positive X-, Y-, Z-axis shown in figure A.1.x correspond to positive Z-, negative Y-, positive Z-axis of a coordinate system commonly used for sensors in mobile operating systems [X12], [X13].

### 5.2.2	Pre/post-processor

An iRTC client in terminal may pre-process the outputs of microphones before they are input to audio encoders, e.g., for limiting bandwidth or converting the output into spatial audio representations. An iRTC client in terminal may post-process the outputs of audio decoders before they are input to speakers, e.g., for acoustically matching the perceived directions or locations of audio with those of video scenes.

### 5.2.3	Codec

Audio codecs for the iRTC client in terminal are specified in [x4], [x7].

## 5.3	Video

### 5.3.1	Camera

An iRTC client in terminal can be connected to one or more color cameras, and/or to one or more depth cameras. Depth cameras in this document typically consist of infrared projectors and infrared cameras that estimate the depth from measured time-of-flight or distortion of projected patterns. Resolutions and frame rates of the cameras are set to meet available bit-rate, complexity, storage, or nature of applications.

The output formats of color cameras, in the form of Y, CR, CB or R, G, B signals, are specified in [x10]. The RGB signals can be input to (2D) video encoders. The output pixel of depth cameras has a value of a 16-bit unsigned number that represents the distance (in millimeters) from the reference point of a depth camera to a point in the captured scene, up to 32.7 meters. The depth signals for a rectangular area (map) can be input to a lossless or lossy encoder, or combined with RGB signals for further processing.

NOTE 1:	With infrared-based depth cameras, measurable distance is typically less than several meters.

NOTE 2:	When the resolutions or aspect ratios of RGB and depth signals differ, the depth signals, whose resolutions are typically lower than those of RGB, can be interpolated to match the RGB signals.

### 5.3.2	Pre/post-processor

An iRTC client in terminal may pre-process the outputs of cameras before they are input to video encoders, e.g., for limiting bandwidth or converting the outputs into other representations, e.g., point cloud. An iRTC client in terminal may post-process the outputs of video decoders before they are input to displays, e.g., for selecting scenes within FoV or enhancing perceived video quality through appropriate filtering.

### 5.3.3	Codec

Video codecs for the iRTC client in terminal are specified in [x4], [x7].

## 5.4	Sensor

An iRTC client in terminal can utilize the information from sensors for understanding environments, processing captured or received media, or other goals. The information can be locally utilized or transmitted with processed media.

### 5.3.1	Measure

## 5.5	Transport protocols

The iRTC client supports transport protocols used in WebRTC, including the protocols for interaction with intermediate boxes such as firewalls, relays, and NAT boxes [x9]. Figure 5.5.x shows the user-plane (right) and control-plane (left) protocol stack.

Figure 5.5.x: Protocol stack for a basic iRTC client

# 6	Session management

## 6.1	General

An iRTC client in terminal interacts with operator networks using 5G system, as illustrated in figure 4.1.x, using WebRTC signaling protocols.

## 6.2	WebRTC signaling protocol

### 6.2.1	General

The Simple WebRTC Application Protocol (SWAP) supports collaboration scenario 3 described in [x5].

NOTE:	The signalling protocol which supports collaboration scenario 4 (and applicable to collaboration scenario 3) is specified as a different protocol in future release.

### 6.2.2	Protocol and version identification

The WebRTC signalling protocol and the version of the protocol shall be determined per WebSocket connection. The WebRTC signalling protocol and the version of the protocol shall be identified by the WebSocket URI for the HTTP upgrade request for WebSocket connection establishment (i.e., the Request-URI of the HTTP request). The WebSocket URI for the HTTP upgrade request shall be consistent with the WebSocket URI structure specified in clause 6.2.3.

The use of "Sec-WebSocket-Protocol" header field is dependent on the WebRTC signalling protocol and the version of the protocol.

### 6.2.3	WebSocket URI structure

WebSocket URI of WebSocket connection for WebRTC signalling protocol message shall be:

{protocolRoot}/<protocolName>/<protocolVersion>

"protocolRoot" shall be a concatenation of the following parts:

-	scheme ("wss")

-	the fixed string "://"

-	authority (host and optional port) as defined in IETF RFC 3986. The host should be represented by the service provider (operator or OTT) specific FQDN (for FQDN examples see clause 28.3.2 in [x18].

-	an optional deployment-specific string (e.g., server prefix) that starts with a "/" character.

"protocolName" shall be protocol-specific string which indicates the name of the WebRTC signalling protocol.

"protocolVersion" shall indicate the version of the WebRTC signalling protocol. The protocol version shall be indicated as the concatenation of the letter "v" and the WebRTC signalling protocol version number. The other fields shall not be included in the URI.

For example, 'v1'.

NOTE:	The "protocolVersion" will only be increased if the new protocol version contains not backward compatible changes.

A URI should not contain a trailing slash, and if it contains one, then it should be ignored/removed.

### 6.2.4	SWAP

#### 6.2.4.1	Protocol and version identification

The SWAP version shall be included in the WebSocket URI path as “/3gpp-swap/v1/".

The present version of SWAP, the Sec-WebSocket-Protocol header field with "3gpp.SWAP.v1" subprotocol identifier shall be included in the HTTP upgrade request.

#### 6.2.4.2	Transport

SWAP protocol shall operate over a full-duplex reliable WebSocket connection between the two endpoints or between an endpoint and a SWAP server. The following figures depict both scenarios.

Figure 6.2.x: Point-to-point SWAP

Figure 6.2.x: SWAP relay

In the former, one of the endpoints shall act as the WebSocket server and listen for the incoming connection request.  The endpoint is not required to support more than one client connection at any point of time.

When a SWAP server is used, sufficient information shall be provided to facilitate the relaying of the messages from the server to the other endpoint.

#### 6.2.4.3	State machine

The SWAP server maintains state information about ongoing WebRTC sessions. The following state machine reflects the state tracked by the SWAP server.

Figure 6.2.x: SWAP state machine

The SWAP protocol is designed to adhere to the JSON Session Establishment Protocol (JSEP) state machine as defined in [x19]. The JSEP state machine is reproduced in the following figure.

Figure 6.2.x: JSEP state machine

SWAP currently does not support preliminary answers in its version 1. Any preliminary answers that are generated by the application will not be sent by the SWAP endpoint.

SWAP version 1 does not support ICE trickling. The final list of ICE candidates is expected to be part of the initial offer message. The application shall wait for the ICE gathering phase to finish prior to sending the offer to the remote endpoint.

#### 6.2.4.4	Message syntax and semantics

##### 6.2.4.4.1	Common message fields

###### 6.2.4.4.1.1	Source (source)

Each message shall carry a unique source identifier that identifies the message source. The source identifier shall be a randomly generated string. The source identifier shall not be changed during the lifetime of a session.

A SWAP server that detects a change in the source identifier from an endpoint over the same WebSocket connection shall ignore the corresponding message. The source identifier shall at least have 10 UTF-8 characters.

###### 6.2.4.4.1.2	Message Identifier (messge_id)

The message identifier shall be a sequence number for the message. The message identifier is scoped by the source identifier, i.e., it shall be uniquely assigned by the source of the message.

The message identifier shall be a positive monotonically increasing number.

###### 6.2.4.4.1.3	Message Type (message_type)

The message type identifies the type of the SWAP message. The supported message types in version 1 of the specification are:

-	Register

-	Response

-	Connect

-	Accept

-	Reject

-	Update

-	Close

-	Application.

The message type shall be considered as a case-insensitive string.

##### 6.2.4.4.2	Register message

###### 6.2.4.4.2.1		Description

An endpoint registers with the SWAP server and provides the matching criteria that may be used to match this endpoint with incoming connection requests.

The register message is not required for the case of a direct connection between the two endpoints.

###### 6.2.4.4.2.2	Parameters

matching_criteria: an object that provides the matching criteria for relaying incoming SWAP messages to their destination. The matching criteria object consists of a type and a value.

The supported types in this version of the specification are the following:

-	ipv4: The IPv4 address of the target endpoint

-	ipv6: The IPv6 address of the target endpoint

-	fqdn: The FQDN of the target endpoint

-	service: An identifier of a service or an application

-	user: An identifier of the user such as a SIP address, a GPSI, or an MSISDN

-	eas: An EAS identifier

-	app: application-specific matching criteria that is compared using binary or string comparison

-	location: one or more identifiers of a geographic location or area

-	qos: a description of the QoS that is supported by the connection to the endpoint

-	processing: a profile description of the processing capabilities of the endpoint.

The matching criteria may be combined together to further restrict the selection of the target endpoint. If multiple endpoints match all provided criteria, then the SWAP server shall randomly select one of the target endpoints.

An endpoint that registers without providing certain matching criteria, such as qos or processing, shall be deprioritized during the selection process, where the request contains these matching criteria.

##### 6.2.4.4.3	Response message

###### 6.2.4.4.3.1	 	Description

A SWAP server shall respond to every received request with a response message. The response message shall indicate whether the message is acknowledged or erroneous.

If a message is relayed properly to an endpoint, an acknowledgement message shall be sent to the source endpoint.

If an error is detected or a target endpoint cannot be identified, the SWAP server shall respond with an error response to the source endpoint.

In addition to the common fields, the response message shall include the request message id. In case of an error response, the message shall contain a textual description of the error.

###### 6.2.4.4.3.2	Parameters

type: the type parameter may either be “ack” or “error”.

target: the identifier of the target of this message, which originated the request message corresponding to this response.

request: the message identifier of the request message that corresponds to this response.

description: in case of an error response, this field provides a description of the error message. In case of an acknowledgement, this description field is optional.

##### 6.2.4.4.4	Connect message

###### 6.2.4.4.4.1		Description

The connect message is used by the source to establish a connection with the endpoint. The request shall include the SDP offer. If connecting via a SWAP server, the request shall include the matching_criteria parameter to identify the target endpoint.

###### 6.2.4.4.4.2	Parameters

offer: a string that includes the SDP description for the offer.

matching_criteria: an array that contains the matching criteria for the target endpoint. Each object shall comply with the definition of a matching criteria as described in clause 6.2.4.4.2.

##### 6.2.4.4.5	Accept message

###### 6.2.4.4.5.1		Description

If the connection request is accepted by the remote endpoint, it shall reply with an accept message. The accept message shall contain the answer SDP.

###### 6.2.4.4.5.2	Parameters

target: This parameter indicates the id of the target endpoint.

answer: This parameter shall contain the answer SDP.

##### 6.2.4.4.6	Update message

###### 6.2.4.4.6.1		Description

The update message may be sent by any of the endpoints of a WebRTC session. It contains the updated SDP, which may add, update, or remove one or more local media streams. If accepted, the remote endpoint shall reply with an accept message.

###### 6.2.4.4.6.2	Parameters

target: This parameter indicates the id of the target endpoint.

sdp: The updated local SDP that is transmitted to the remote endpoint.

##### 6.2.4.4.7	Reject message

###### 6.2.4.4.7.1		Description

In case the remote endpoint does not accept the offer or update message, it shall respond with the reject message. The message shall contain a reference to the corresponding offer or update message as well as a description of the reason why the message was rejected.

###### 6.2.4.4.7.2	Parameters

target: this parameter indicates the id of the target endpoint

request: the message identifier of the request

error_id: an identifier of the error message

description: a description of the error message.

##### 6.2.4.4.8	Close message

###### 6.2.4.4.8.1		Description

The close message may be triggered by any of the two endpoints of a WebRTC session. Upon reception, the endpoint shall respond with an accept message, after which the WebRTC session is torn down and the resources associated with the WebRTC session are released.

###### 6.2.4.4.8.2	Parameters

target: this parameter indicates the id of the target endpoint

###### 6.2.4.4.9	Application message

###### 6.2.4.4.9.1		Description

Application-specific message may be defined by the application and exchanged between the endpoints of a WebRTC session. The message shall contain a type that uniquely identifies the type of the application message. If an application message type is not supported, it shall be rejected by the remote endpoint.

###### 6.2.4.4.9.2 	Parameters

target: this parameter indicates the id of the target endpoint

type: the type of the application message shall be a URN that uniquely identifies the application message type.

value: an object that contains the application message content.

#### 6.2.4.5		Integrity and security

Integrity and confidentiality protection are supported through the protection of the message information as follows:

-	A key derivation mechanism is configured by the application provider to the session participants, e.g., using a shared secret algorithm

-	For integrity protection, the derived key is used to provide integrity protection, e.g., using a Message Authentication Code (MAC) for message payload

-	For encryption, the derived key is used to encrypt the message payload. The encrypted data may then be encoded using base64 to enable embedding it in JSON.

These mechanisms are possible to implement using the WebCrypto API, which makes them web-friendly. Consulting with SA3 on these security algorithms is recommended.

#### 6.2.4.6		JSON schema

The JSON schema of the SWAP messages is follows:

{ "$schema": "http://json-schema.org/draft-07/schema", "title": "3GPP.SWAP", "type": "object", "description": "The description of the SWAP messages", "properties": { "version": { "description": "the version of the SWAP protocol", "type": "integer" }, "source_id": { "description": "A unique identifier of the source", "type": "string" }, "message_id": { "description": "the sequence number of the message ", "type": "integer" }, "message_type": { "description": "the type of the SWAP message", "type": "string", "enum": ["register", "connect", "response", "accept", "reject", "update", "close", "application"] }, "oneOf": [ { "type": "object", "properties": { "matching_criteria": {"type": "string", "enum": ["ipv4", "ipv6", "fqdn", "service", "user", "eas", "app", "location", "qos", "processing"]} } }, { "type": "object", "properties": { "type": {"type": "string", "enum": ["ack", "error"]}, "source": {"type": "string"}, "request": {"type": "integer"}, "description": {"type": "string"} } }, { "type": "object", "properties": { "offer": {"type": "string"}, "matching_criteria": {"type": "string", "enum": ["ipv4", "ipv6", "fqdn", "service", "user", "eas", "app", "location", "qos", "processing"]} } }, { "type": "object", "properties": { "answer": {"type": "string"} } }, { "type": "object", "properties": { "source": {"type": "string"}, "request": {"type": "number"}, "error_id": {"type": "string"}, "description": {"type": "string"} } }, { "type": "object", "properties": { "type": {"type": "string"}, "value": {"type": "object"} } } ], "extensions": {} }, "required": ["version", "source", "message_id"] }

#### 6.2.4.7	Protocol operation

SWAP is an acknowledged signaling protocol for WebRTC. Each message that the WebRTC signaling server receives shall be acknowledged after proper processing. This is valid for the case where one of the endpoints acts as the signaling server. The Response message may also indicate an error, in case the received message can not be processed and forwarded properly.

The error messages shall be formatted according to the Problem Details specification in RFC7807. The following error message types are defined in this specification:

| Error message type | Error message title |
| --- | --- |
| http://forge.3gpp.org/sa4/swap/message_unknown.html | Message type unknown |
| http://forge.3gpp.org/sa4/swap/message_malformatted.html | Message malformatted |
| http://forge.3gpp.org/sa4/swap/target_unknown.html | Target cannot be located |
| http://forge.3gpp.org/sa4/swap/unauthorized.html | Unauthorized |

The WebRTC Signaling Function uses the (source, target) identifier pairs of the communicating endpoints to identify the session and properly route the messages. Note that in the first connect message, the target identifier might not be known; in which case, the routing is done based on the matching criteria.

The source identifier shall be a string that uniquely identifies the source. An example of such identifier may be a randomly generated UUID.

Every message shall contain the common message fields: source, message_id, and message_type. The source field shall always indicate the originator of the current message. A WebRTC signaling server shall also generate and use a unique identifier.

# 7	Inter-working

## 7.1	General

[Editor’s note: description of inter-working an iRTC client in terminal with another client connected to 3GPP and non-3GPP networks, including tethering]

# 8	Packet-loss handling

## 8.1	General

[Editor’s note: description of measures for link quality degradation, e.g., in poor channel condition, overloading, etc]

# 9	Implementor's guide

## 9.1	General

[Editor’s note: generic guidelines for configuring & operating an iRTC client in terminal]

Annex A (informative):

[Editor’s note: ]

# A.1	General

[Editor’s note: ]

Annex B (informative):

[Editor’s note: ]

# B.1	General

[Editor’s note: ]

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-05 | SA4#119 | S4-220768 |  |  |  | Skeleton for TS 26.113 | 0.1.0 |
| 2022-11 | SA4#121 | S4-221275 |  |  |  | Skeleton for TS 26.113 | 0.2.0 |
| 2023-04 | SA4#123e | S4-230590 |  |  |  | WebRTC Signaling Protocol (SWAP) | 0.5.0 |
| 2023-04 | SA4#123e | S4-230651 |  |  |  | Functional components of iRTC client in terminal | 0.5.0 |
| 2023-04 | SA4#123e | S4-230654 |  |  |  | High-level architecture, microphone description and transport protocol stack | 0.5.0 |
| 2023-05 | SA4#124 | S4-230747 |  |  |  | Editor’s update | 0.6.0 |
| 2023-05 | SA4#124 | S4-230980 |  |  |  | Updates to SWAP protocol | 0.6.0 |