| 3GPP TR 26.819 V1.1.0 (2025-07) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Multimedia Codecs Systems and Services; Study on Spatial Computing for Augmented Reality (AR) Services (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Abbreviations	10

4	Introduction to spatial computing	10

4.1	Spatial computing functions	10

4.1.1	General	10

4.1.2	World tracking	11

4.1.3	Relocalization	12

4.1.3.1 	 Introduction	12

4.1.3.2	 Example: Niantic Lightship™	13

4.1.3.3	 Example: Augmented.City	14

4.1.3.4	 Example: Google ARCore™ Geospatial API	14

4.1.3.5	 Example:  b<>com *Overview*	15

4.1.4	Anchoring	15

4.1.4.1	 Introduction	15

4.1.4.2	Example: Qualcomm™ Snapdragon™ Spaces	17

4.1.5	3D model reconstruction and retrieval	18

4.1.5.1	 Introduction	18

4.1.5.2	 Example: Microsoft™ HoloLens™	19

4.1.5.3	 Example: Qualcomm™ Snapdragon™ Spaces	20

4.1.5.4	Example: Magic Leap™ AR Cloud	20

4.1.6	Segmentation and labelling	20

4.1.6.1	 Introduction	20

4.1.6.2	 Example: Meta™ Space Setup	21

4.1.6.3	 Example: Apple™ RoomPlan API	21

4.1.6.4	 Example: Google™ Scene Semantics API	22

4.1.7	Collider generation	23

4.1.8	Light extraction	24

4.1.9	Semantic perception	25

4.1.10	Real object removal	26

4.1.11	Summary of spatial description formats	27

4.2	Quality of experience (QoE) for spatial computing	30

5	Related standardization work	31

5.1	3GPP	31

5.2	Non-3GPP	31

5.2.1	Khronos	31

5.2.2	ETSI	31

5.2.3	MPEG	32

5.2.4	Open Geospatial Consortium (OGC)	33

5.2.5	Open AR Cloud	33

6	Mapping of spatial computing to 5G services	34

6.1	Introduction	34

6.2	Terminal Architecture	34

6.3	General Architecture for Spatial Computing	35

6.4 	Call flow for spatial computing session setup and operation	36

6.5	Cross operation with other architectures and media service enablers	37

6.5.1	Extended Spatial Computing architecture for Edge Computing	37

6.5.1.1	General	37

6.5.1.2	Edge Application Server (EAS)	38

6.5.1.3	Edge Interfaces	39

6.5.2	Potential Prerequisites for a Spatial Computing MSE	39

6.5.2.1	General	39

6.5.2.2	Pre-requisites on 5G System	39

6.5.2.3	Pre-requisites on Device APIs and Functionality	39

6.5.2.4	Pre-requisites on Network Support	40

6.5.2.5	Pre-requisites on Edge Resources	40

6.5.3	Potential mapping to Generalized IMS DC Architecture	40

6.5.3.1	General	40

6.5.3.2	Call flow for session set up and operation	41

7	Conclusions and proposed next steps	43

7.1	Conclusions	43

7.2	Proposed next steps	44

Annex A: Spatial computing use cases	45

A.1	Environment Aware AR Gaming	45

A.1.1	Description	45

A.1.2 	Potential use of spatial computing functions	45

A.2	AR IoT control and supervision	46

A.2.1	Description	46

A.2.2 	Potential use of spatial computing functions	47

A.3	Diminished reality	47

A.3.1	Description	47

A.3.2 	Potential use of spatial computing functions	48

A.4	XR Conference	48

A.4.1	Description	48

A.4.2 	Potential use of spatial computing functions	50

Annex B: Change history	51

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document documents a set of common spatial computing functions and relevant quality-of-experience metrics for some of these functions. It also provides potential mappings to and cross-operation procedures with 3GPP architectures and recommendations on relevant aspects for defining a media service enabler for spatial computing.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 26.928: "Extended Reality (XR) in 5G".

[3]	3GPP TR 26.998: "Support of 5G glass-type Augmented Reality / Mixed Reality (AR/MR) devices".

[4] 	ARCore™ SLAM, https://developers.google.com/ar/develop/fundamentals

[5] 	ARKit™ VIO, https://developer.apple.com/documentation/arkit/arkit_in_ios/configuration_objects/understanding_world_tracking

[6]	ARCore™ Cloud Anchor, https://codelabs.developers.google.com/codelabs/arcore-cloud-anchors#0

[7]	ARKit™ World Map, https://developer.apple.com/documentation/arkit/arworldmap

[8]	Meta™ Quest Spatial Anchor, https://developer.oculus.com/documentation/unity/unity-spatial-anchors-overview/

[9]	HoloLens, https://learn.microsoft.com/en-us/windows/mixed-reality/design/spatial-mapping

[10]	Meta™ Space Setup, https://www.uploadvr.com/quest-v64-undocumented-features-furniture-recognition-multimodal/

[11]	Apple™ RoomPlan, https://developer.apple.com/augmented-reality/roomplan/

[12]	Google™ Scene Semantics, https://developers.google.com/ar/develop/scene-semantics

[13]	Unity Documentation - Collision, https://docs.unity3d.com/2023.1/Documentation/Manual/collision-section.html

[14]	Blender Documentation – Collisions, https://docs.blender.org/manual/en/latest/physics/rigid_body/properties/collisions.html

[15]	Unity Documentation - Compound Colliders, https://docs.unity3d.com/Manual/compound-colliders-introduction.html

[16]	Unity Documentation - Mesh Colliders, https://docs.unity.cn/Manual/mesh-colliders-introduction.html.

[17]	ARCore™ Ligthing Estimation, https://developers.google.com/ar/develop/lighting-estimation

[18]	Cast Shadow https://ieeexplore.ieee.org/document/9018202

[19]	Macario Barros, A., Michel, M., Moline, Y., Corre, G., & Carrel, F. (2022). A comprehensive survey of visual slam algorithms. Robotics, 11(1), 24.

[20]	Khronos OpenXR™: https://registry.khronos.org/OpenXR/specs/1.0/html/xrspec.html

[21]	ETSI Industry Specification Group AR Framework (ISG ARF), https://www.etsi.org/committee/1420-arf

[22]	ETSI GS ARF 004-2: “Augmented Reality Framework (ARF); Interoperability Requirements for AR components, systems and services - Part 2: World Storage and AR Authoring functions”.

[23] 	ETSI GS ARF 005: “Augmented Reality Framework (ARF); Open APIs for the Creation and Management of the World Representation”.

[24]	ISO/IEC 23090-14, Text of ISO/IEC FDIS 23090-14 2nd edition Scene description, April 2024.

[25]	3GPP TS 26.119: “Media Capabilities for Augmented Reality”

[26]	3GPP TS 26.143: “Messaging Media Profiles”

[27]	3GPP TS 26.264: “IMS-based AR Real-Time Communication”

[28]	3GPP TR 26.812: “QoE metrics for AR/MR services”

[29]	3GPP TR 23.700-21: Study on Application architecture for enabling mobile metaverse applications.

[30]	Open Geospatial Consortium: https://www.ogc.org/

[31] 	Open AR Cloud: https://www.openarcloud.org/

[32]	ETSI GS ARF 004-4: “Augmented Reality Framework (ARF); Interoperability Requirements for AR components, systems and services - Part 4: World Analysis, World Storage and Scene Management functions”.

[33]	OGC GeoPose 1.0 Data Exchange Draft Standard, https://docs.ogc.org/dis/21-056r10/21-056r10.html.

[34]	OGC Points of Interest Conceptual Model Standard (draft document), https://docs.ogc.org/DRAFTS/21-049.html

[35]	3GPP TR 22.856: "Feasibility Study on Localized Mobile Metaverse Services".

[36]	SIFT, https://www.vlfeat.org/api/sift.html

[37] 	E. Rublee, V. Rabaud, K. Konolige and G. Bradski, "ORB: An efficient alternative to SIFT or SURF," 2011 International Conference on Computer Vision, Barcelona, Spain, 2011, pp. 2564-2571.

[38] 	A. Alahi, R. Ortiz and P. Vandergheynst, "FREAK: Fast Retina Keypoint," 2012 IEEE Conference on Computer Vision and Pattern Recognition, Providence, RI, USA, 2012, pp. 510-517.

[39]	M. Calonder, V. Lepetit, M. Ozuysal, T. Trzcinski, C. Strecha and P. Fua, "BRIEF: Computing a Local Binary Descriptor Very Fast," in IEEE Transactions on Pattern Analysis and Machine Intelligence, vol. 34, no. 7, pp. 1281-1298, July 2012.

[40] 	OBJ, https://en.wikipedia.org/wiki/Wavefront_.obj_file

[41] 	PLY, http://gamma.cs.unc.edu/POWERPLANT/papers/ply.pdf

[42] 	FBX, https://www.autodesk.com/products/fbx/overview

[43] 	STL, https://docs.fileformat.com/cad/stl/

[44] 	glTF, https://registry.khronos.org/glTF/specs/2.0/glTF-2.0.pdf

[45] 	USD, https://openusd.org/dev/api/class_usd_object.html

[46]	Unity, https://docs.unity3d.com/Manual/mesh-colliders-introduction.html

[47] 	ISO/IEC 23090-14, Text of ISO/IEC FDIS 23090-14 2nd edition Scene description, April 2024.

[48]	FAST, http://www.edwardrosten.com/work/rosten_2006_machine.pdf

[49]	Herbert Bay, Andreas Ess, Tinne Tuytelaars, Luc Van Gool, Speeded-Up Robust Features (SURF), Computer Vision and Image Understanding, Vol. 110, Issue 3, 2008, pp. 346-359.

[50]	3GPP TS 23.558: “Architecture for enabling Edge Applications”

[51]	3GPP TS 26.501: “5G Media Streaming (5GMS); General description and architecture”

[52]	3GPP TS 23.548: “5G System Enhancements for Edge Computing; Stage 2”

[53]	3GPP TS 23.501: “System architecture for the 5G System (5GS)”

[54]	W3C WebXR Device API: https://immersive-web.github.io/webxr

[55]	3GPP TS 23.503: “Policy and charging control framework for the 5G System (5GS); Stage 2”

[56]	Niantic Visual Positioning System: https://www.nianticspatial.com/products/visual-positioning-system

[57]                     Niantic™ Lightship VPS: https://lightship.dev/docs/ardk/features/lightship_vps/

[58]                     Augmented.City: Augmented City API

[59]                     Google™ Geosptial API: https://developers.google.com/ar/develop/geospatial

[60]                     bcom<>*Overview*: https://b-com.com/en/overview

[61] 	Niantic™ Lighship Localizing with VPS: https://www.lightship.games/docs/ardk/vps/vps_localization.html

[62]	Magic Leap™ AR Cloud: http://magicleap.care/hc/en-us/articles/9312806819597-AR-Cloud

[63]	3GPP TS 23.228: “IP Multimedia Subsystem (IMS); Stage 2”.

[64]	3GPP TS 23.501: “System architecture for the 5G System (5GS)”.

[65]	3GPP TS 26.506: “5G Real-time Media Communication Architecture (Stage 2)”

[66]	3GPP TS 23.437: “Service Enabler Architecture Layer for Verticals (SEAL); Spatial map and Spatial anchors”

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

spatial computing: AR functions which process sensor data to generate information about the world 3D space surrounding the AR user.

spatial mapping: The process of mapping real-world surfaces into the virtual world.

XR spatial description: a data structure describing the spatial mapping of the real-world using anchors, trackables, camera parameters and visual features.

XR Space: as defined in TS 26.119.

anchor: as defined in TS 26.119.

trackable: as defined in TS 26.119.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

MSE	Media Service Enabler

# 4	Introduction to spatial computing

## 4.1	Spatial computing functions

### 4.1.1	General

XR spatial computing encompasses a set of functions which process sensor data to generate information about the world 3D space surrounding the AR user. These include functions such as relocalization (establishing the position of users and objects within that space), anchoring, 3D Model reconstruction, segmentation and labeling (semantic perception), light extraction, and real object removal are achieved through a combination of advanced sensors, cameras, and algorithms that enable devices to understand and interact with the three-dimensional space around them.

This requires accurately localizing the AR device worn by the end-user in relation to a spatial coordinate system of the real-world space. Vision-based localization systems reconstruct a sparse spatial mapping of the real-world space in parallel (e.g., SLAM). Beyond the localization within a world coordinate system, which is usually based on a sparse spatial map, dense spatial mapping of objects is also essential in order to place 3D objects on real surfaces and provides the ability to occlude objects behind surfaces, do physics-based interactions based on surface properties, provide navigation functions, or provide a visualization of the surface.

For the purpose of understanding and perceiving the scene semantically, machine-learning and/or artificial intelligence may be used to provide context for the observed scene.

The output of spatial computing is spatial mapping information that is organized in a data structure called the XR Spatial Description for storing and exchanging the information. Some spatial computing functions may also take an XR Spatial Description and may result in updates to the XR Spatial Description. Spatial computing functions typically include data exchange and require a network architecture.

An AR device may provide sensor data to a spatial computing function to create or update the spatial mapping information. The device may also access a spatial computing function to retrieve different spatial mapping information depending on the needs of the XR application.

The spatial computing functions can run locally on the AR device or can be executed remotely on the cloud or on the edge in a spatial computing server accessed through dedicated interfaces as detailed in TR 26.298 [2].

The main functions provided by a spatial computing service are given in Figure 4.1.1-1and explained in the following subclauses.

Figure 4.1.1-1: Spatial computing functions

### 4.1.2	World tracking

Simultaneous Localization and Mapping (SLAM) is a technology that allows an AR device to create a map of its environment while simultaneously determining its own location within that map. The goal of SLAM is to obtain a global, consistent estimate of the camera path. The map of the environment is usually kept for helping localization. SLAM achieves its objectives using two parts: (1) a tracking mechanism, which solves the localization problem by comparing the new input data with the currently existing map, and (2) a mapping mechanism, which supports the tracking by providing, maintaining and expanding the map itself based on the information obtained from the input data obtained by the tracking part. Loop detection is the process of identifying when a device has returned to a previously visited location. When a loop is detected, this information is used to reduce the drift in both the map and pose. Loop closure is the process of correcting the device’s pose and map based on the detected loop. This involves adjusting the device’s trajectory and the map to ensure consistency and eliminate drift. Beside localization, loop detection and loop closure are two main challenges in SLAM.

The visual-based approaches can be divided into three main categories: visual-only SLAM, visual-inertial SLAM, and RGB-D SLAM [19].

Visual-only SLAM refers to the techniques based only on 2D images provided by a monocular or stereo camera. This type of SLAM uses images captured from one or more cameras to detect features in the environment, track their movements across frames, and estimate the motion of the camera and the structure of the environment. The primary challenge in visual-only SLAM is dealing with scenarios where visual information is poor, such as in low-light conditions or in textureless environments. It is also susceptible to accumulating drift over time since it only relies on visual data. Robustness of the algorithms can be increased by adding information from an inertial measurement unit (IMU). Visual-inertial SLAM techniques combine data from cameras and inertial sensors (accelerometers and gyroscopes) to estimate the pose of the device. RGB-D SLAM techniques employ a depth sensor and leverage depth data alongside visual data, which simplifies the mapping process but can be limited by the operational environment of the depth sensors (more suitable to indoor environments).

Another algorithm used for world tracking is the visual inertial odometry which combines information from the device’s motion sensing hardware and computer vision analysis of the scene visible to the device’s camera. The process recognizes notable features in the scene image, tracks differences in the positions of those features across video frames and compares that information with motion sensing data. The result is a high-precision model of the device’s position and motion.

The main difference between visual SLAM and visual odometry lies in considering, or not, the global consistency of the estimated trajectory and map. While visual odometry performs only local optimizations, visual-SLAM algorithms also employ loop closure detection [19].

By aligning the pose of the virtual camera that renders the 3D content with the pose of the device's camera, virtual content can be rendered from the correct perspective. The rendered virtual image can be overlaid on top of the image obtained from the device's camera, making it appear as if the virtual content is part of the real world.

As the AR device moves through the world, SLAM is used to understand where the device is relative to the world around it. The process detects visually distinct features in the captured camera image called feature points (=feature map) and uses these points to compute a change in location. The visual information is combined with inertial measurements from the device's IMU to estimate the pose (i.e., position and orientation) of the camera relative to the world over time.

ARCore supports world tracking using visual-inertial SLAM [4], while ARKit uses Visual Inertial Odometry [5].

For world tracking, the following input data may be used:

-	Sensor data:

- Images captured by AR Device

- IMU data

- pose of AR Device

- Depth maps

The output of the World Tracking function is a set of features (Feature Map). Features are extracted from images and described using descriptor. Common feature detection algorithms include: the features from accelerated segment test (FAST) [48] algorithm, the speeded-up robust features (SURF) [49] algorithm, the oriented FAST and rotated BRIEF (ORB) [37] algorithm, and the scale invariant feature transform (SIFT) [36] algorithm.

Common feature descriptors include:

- SIFT – Scale Invariant Feature Transform [36]

- ORB – Oriented FAST Rotated BRIEF (Binary descriptor) [37]

- FREAK – Fast Retina Keypoint (Binary descriptor) [38]

- BRIEF – Binary Robust Independent Elementary Features (Binary descriptor) [39]

- SURF – Speeded-Up Robust Features [49]

### 4.1.3	Relocalization

#### 4.1.3.1 		Introduction

Relocalization is a function that estimates the pose of the AR device according to a known real environment. It is used to estimate the pose of the AR device at initialization, when tracking is lost, or regularly to correct the drift of the tracking (TR 26.998 [3] clause 4.2.3). Cameras capture the real world, while sensors (accelerometers, gyroscopes, and depth sensors) contribute additional data for mapping and positioning. Computer vision algorithms process this data to determine the location and orientation of the device relative to its environment.

SLAM, Visual Localization, e.g., Visual SLAM (vSLAM), or Visual Positioning System (VPS) are all algorithms that can be used for mapping unknown environments while also maintaining the localization of the device/user within that environment, as explained in TR 26.928 [2] clause 4.1.4.

Visual Positioning System (VPS), an example of relocalization, relies on network services to provide the mapping information. Such functions may be executed in the network for computing and storage capability reasons. In such a case, the VPS relies on pre-existing mapping of the real environment. The previously mentioned sensor data can be sent to a server which then computes and provides the pose of the AR device in the reference coordinate system of a known real environment. In an alternative implementation, the AR device retrieves a local map from the server based on this GPS position and then performs the relocalization computation locally.

Combining a spatial description of the real world with the estimation of the pose of different trackables can also be used for localization as proposed with the World Storage and World Analysis components of the ETSI ARF architecture [21].

For relocalization, the following input data may be used:

-	Sensor data:

-	images (for SLAM)

-	depth maps

-	GNSS data

-	local pose of AR Device

- Derived data

-	extracted feature vectors

The output of the relocalization function is a pose, which is defined by a position and an orientation.

Examples of VPS include Google GeoSpatial [59] and Niantic Lightship [56] APIs. Standardized interfaces between AR application and remote relocalization services have been proposed by ETSI ARF as described in Section 5.2.2 and by the Open Spatial Computing Platform (OSCP) with the GeoPoseProtocol API. The GeoPoseProtocol API provides an exhaustive list of these exchanged data between an AR client and a remote relocalization server.

Regarding QoS, the two important aspects are:

- The network capacity to transfer high resolution images from the client to the server at regular intervals. For instance, transferring two 1280×720 images (depth and RGB) per second in a JPEG format could require between 8 and 16 Mbps of bandwidth. Considering that the AR device has reliable World Tracking system, relocalization can be done at larger intervals releasing the need for a high bandwidth.  As an example, Niantic Lighship VPS requires the client to send around 300Kb of data to the server every 1-3 seconds [61] therefore requiring a bandwidth between 0.82 and 2.46 Mbps.

- The end-to-end latency of the relocalization process includes the capture of the data from the device sensors, their transmission through the network, their processing on the server, and the transmission of the pose from the server to the user device. With XR devices, motion-to-photon latency is expected to be between 10 and 15 milliseconds to avoid any cybersickness effect. However, with a World Tracking system running locally on the AR device of the client, there is no strong latency constraint, as the pose received from the VPS can be corrected thanks to the difference between the local poses obtained when sending data and when receiving relocalization data. Keeping latency as low as possible (<100ms) still limits potential drifting issues.

#### 4.1.3.2		Example: Niantic Lightship™

Lightship™ is Niantic's toolkit for creating immersive location-based experiences. Niantic's Visual Positioning System (VPS) [57] is a cloud-based system that allows devices to know where they are relative to the Lightship™ map. By determining the six-degrees-of-freedom pose of a device relative to the map, you can place persistent virtual objects that stay aligned with the real world.

The Lightship™ VPS map is built using scans submitted by users and is organized by the coverage of the locations in the scans. When enough scans have been submitted for a location, the location becomes "VPS-Activated". The highest-quality space of a VPS-Activated location will be saved as the default space for that location with each default space having a default anchor associated with it.

The VPS Coverage API provides functionality for discovering AR Locations at runtime to display them on maps and use them as localization targets. The VPS Coverage API provides the default anchor as part of the LocalizationTarget.

To localize, the user needs to point their device at the real-world location associated with that anchor. Lightship™ then sends the camera frames to the cloud and looks for a match.

#### 4.1.3.3		Example: Augmented.City

Augmented.City [58] is an augmented reality cloud and platform ecosystem that allows users to capture, enrich with data, and visualize it on location in basically any device.

With the Augmented.City platform, users can easily create city-scale AR Clouds, populate them with information, and instantaneously localize inside them. The information could be visualized and shared with any mobile device.

To get started, the user needs to localize in a previously scanned location, point the camera at the nearest facade and start AR mode.

The user sends a request with sensor data and receives a pose. An example request and response are given in Table 4.1.3.3-1.

Table 4.1.3.3-1: Augmented.City relocalization request-response example.

| Sample Request | Sample Response |
| --- | --- |
| { "id": "string", "timestamp": 0, "type": "string", "sensors": [{}], "sensorReadings": [{}], "priorPoses": [{}], "hint": { "reconstructions": [], "hint_only": false } } | { "id": "string", "timestamp": 0, "accuracy": { "position": 0, "orientation": 0 }, "type": "string", "geopose": { "position": {}, "quaternion": {} }, "ecefPose": { "position": {}, "orientation": {} }, "localPose": { "position": {}, "orientation": {} }, "reconstruction_id": 390 } |

#### 4.1.3.4		Example: Google ARCore™ Geospatial API

The ARCore™ Geospatial API [59] enables the user to remotely attach content to any area covered by Google Street View™ and creates AR experiences on a global scale. It uses device sensor and GPS data to detect the device's environment, then matches the recognizable parts of that environment to a localization model provided by Google’s Visual Positioning System (VPS) to determine the precise location of a user’s device.

Street View images from Google Maps™, which have been captured around the globe for more than 15 years, are the foundation of VPS. Deep neural networks identify and describe parts of the images that are likely to be recognizable over long periods of time. Those parts are then combined across tens of billions of images to compute a 3D point cloud of the global environment. This localization model consists of trillions of points and spans nearly all countries, with future coverage.

When the user’s device makes a request to the Geospatial API, a neural network processes the pixels to find recognizable parts of the user’s environment and matches them to the VPS localization model. Computer vision algorithms then compute the position and orientation of the device, offering a location that is much more accurate than what was previously possible with GPS alone.

#### 4.1.3.5		Example:  b<>com *Overview*

b<>com *Overview* [60] enables secure and accurate augmented reality experiences running on any device. It consists of a set of software services able to map the real environment and locate in 3D AR devices (smartphones, tablets, and AR glasses) at a building, construction site, or factory scale.

Spatial computing services supported by b<>com *Overview* run on GPU servers. For relocalization, b<>com *Overview* uses a single visual positioning system. Using a Unity plugin, an AR Client can connect an application to the b<>com *Overview* services.

### 4.1.4	Anchoring

#### 4.1.4.1		Introduction

As defined in TS 26.119, a trackable is a real-world object that can be tracked by the XR runtime. Each trackable provides a local reference space, also known as a trackable space, in which an anchor can be expressed. Examples of trackables that AR runtimes can track include:

- Image markers: This includes fiducial markers (such as QR codes) and natural image markers

- Meshes: These can be scanned with a dedicated tool or stored in specific formats (CAD for instance)

- Body parts: This encompasses user skeletons and facial bones.

- Planes: Flat surfaces in the physical environment that come with both tracking and shape information.

- Geo-trackables: A Geo Trackable is a position and orientation on earth in a geodetic referential.

ARKit™ and ARCore™ both support image, geo-trackable, body, and plane detection and tracking. An example of body part tracking (here the face) with ARCore™ is shown in Figure 4.1.4.1-1. ARKit™ also supports meshes as demonstrated in Figure 4.1.4.1-2.

Figure 4.1.4.1-1: ARCore™ face tracking (Source: https://developers.google.com/ar/develop/augmented-faces, Google™)

Figure 4.1.4.1-2: ARKit™ 3D mesh tracking (Source: https://developer.apple.com/documentation/arkit/scanning-and-detecting-3d-objects, Apple™)

Like trackables, spatial anchors are a concept in Augmented Reality which serves as fixed reference points that AR devices can detect and use to position virtual content accurately relative to the real-world coordinates. Virtual objects or information are anchored to specific locations in physical space. But Spatial Anchors have additional functionalities enabling the persistence and stability of virtual content in the real world.

A spatial anchor acts as a marker or reference point in the real world that AR devices can recognize and track. Spatial anchors are created by leveraging spatial mapping techniques, which involve capturing and analyzing the physical features of the surrounding environment. This can be done through depth-sensing cameras, LiDAR scanners, or other sensors to understand the geometry and spatial characteristics of the space. These techniques help applications identify stable, distinctive features in the space, allowing precise placement of anchors that can persist and be tracked across different sessions.

Applications can use one anchor per virtual object or choose to have multiple virtual objects use the same anchor.

Once the spatial mapping is performed, spatial anchors are placed at desired locations within the mapped environment. Anchor can be persistent for the entire duration of the session.

The three main AR SDK (Meta™, Google™ and Apple™) offer anchoring solution:

-	ARCore™, Cloud Anchor [6]

-	ARKit™, World Map [7]

-	Meta Quest™, Spatial Anchor [8]

For creating spatial anchors, the following input data can be used:

-	Anchor pose for creation

-	Sensor data:

-	images

-	pose of AR device

-	depth data (e.g., LiDAR)

The Anchoring function provides several services:

- Creation: when a new trackable is created, information is extracted from the trackable and an anchor is created with a relative pose to the trackable. A unique identifier of the newly created trackable is returned.

- Consumption: from a trackable ID, the function retrieves the stored trackable and all of its information. Sensor data and the features of the trackable are used to compute the pose of the anchor(s) associated with the trackable. When the trackable is detected, the anchors are positioned, and their poses are returned.

- Synchronization: This function enables spatial anchors to be synchronized across multiple AR devices, allowing for shared AR experiences, e.g. for collaborative applications where multiple users interact with the same virtual content.

- Persistence: This function enables spatial anchors to be saved and retrieved across sessions. This means that virtual objects can remain in the same physical location even after the AR application is closed and reopened.

#### 4.1.4.2	Example: Qualcomm™ Snapdragon™ Spaces

The Snapdragon™ Spaces platform by Qualcomm™ offers a suite of APIs for environmental understanding, tailored for developers using Unity™ and Unreal™. These APIs enable applications to detect, interact, and anchor AR elements to real-world surfaces, images, and structures.

The API allows applications to identify horizontal and vertical surfaces, providing essential data for placing virtual objects accurately in the environment. In Unity, developers use the `PlaneManager` component to access these detections.

// Initialize plane detection PlaneManager planeManager = new PlaneManager(); // Start detecting planes (horizontal and vertical) planeManager.RequestPlaneDetection(PlaneDetectionMode.Both); // Check detected planes foreach (var plane in planeManager.DetectedPlanes) { Vector3 planeCenter = plane.Center; Vector3 planeNormal = plane.Normal; Debug.Log($"Detected Plane - Center: {planeCenter}, Normal: {planeNormal}"); }

In this code snippet, RequestPlaneDetection initiates detection for both horizontal and vertical planes. Each detected plane has attributes like Center (position) and Normal (orientation). The PlaneManager provides real-time updates, which developers can use to position virtual objects aligned with real surfaces.

Snapdragon™ Spaces’ image tracking API lets applications recognize and track specific image targets, allowing for precise augmentation on top of the tracked images. The image tracking API requires loading reference images as targets and then monitoring their position and orientation.

// Load image target with an identifier ImageTracker imageTracker = new ImageTracker(); imageTracker.LoadImageTarget("targetImage", "path/to/image.jpg"); // Access tracking data in the update loop if (imageTracker.GetTrackingState("targetImage") == TrackingState.Tracked) { Pose imagePose = imageTracker.GetImagePose("targetImage"); Vector3 imagePosition = imagePose.position; Quaternion imageRotation = imagePose.rotation; Debug.Log($"Tracked Image - Position: {imagePosition}, Rotation: {imageRotation}"); }

In this code, LoadImageTarget loads an image by specifying a path and an identifier targetImage. GetTrackingState returns the current tracking state, with `Tracked` indicating active tracking. The GetImagePose method provides the image’s position and orientation in the environment, enabling content to be dynamically aligned to it.

In addition to image tracking, The Snapdragon™ Spaces API also supports QR code identification and tracking in the real-world environment, which can trigger AR content or experiences based on specific codes. Developers can configure QR code recognition parameters and retrieve positional and orientation data.

// Load and initialize QR code tracking QRCodeTracker qrTracker = new QRCodeTracker(); qrTracker.StartTracking(); // Process tracked QR codes foreach (var qrCode in qrTracker.TrackedCodes) { string qrData = qrCode.Data; Vector3 qrPosition = qrCode.Pose.position; Quaternion qrRotation = qrCode.Pose.rotation; Debug.Log($"QR Code - Data: {qrData}, Position: {qrPosition}, Rotation: {qrRotation}"); }

In this snippet, StartTracking initializes QR code detection. Each tracked QR code provides Data, which holds the QR code’s information, and Pose, representing its position and orientation in the environment. These details can be used to dynamically trigger content based on the recognized QR code.

### 4.1.5	3D model reconstruction and retrieval

#### 4.1.5.1		Introduction

Spatial computing enables the creation of accurate 3D models of surrounding space. It accurately captures real-world scenes and objects using 3D scanning techniques or photogrammetry. These 3D models can be displayed in immersive 3D environments in real-time to provide users with a sense of interactivity and presence. When built in real-time, they can, for example, be used to manage occlusion and physics behaviors between real and virtual objects. Such 3D models can also be used offline for authoring AR experiences, and for completing or correcting a digital twin of a real environment.

The 3D model of a real-world environment may also be constructed collectively by aggregating meshes captured by an AR device.

HoloLens™ from Microsoft™ and AR SDKs from Apple™ and Meta™ can all build a 3D model of the surrounding environment in real-time.

Achieving highly realistic and accurate 3D model reconstruction can require significant computing capabilities, especially when dealing with multiple users capturing the same space, and when using advanced structure-from-motion algorithms or radiance-field-based representations, such as Gaussian splatting. This can be an issue for some devices. These algorithms can then be executed offline on remote servers using sensor data provided by one or multiple AR devices, with the reconstructed models sent back upon completion.

To build the 3D model, the following input data can be used:

- Sensor data:

- Images captured by AR Device

- pose of AR Device

- Depth map (image or texture)

- Mesh captured by AR Device

The output of the 3D Model construction is a 3D Model. A 3D model is a broader term that encompasses the complete representation of a 3D object, including its geometry (which may be represented by a mesh), texture, and materials.

When using a remote computing server, streaming all these data from several AR client can require a high bandwidth. As an example, considering an RGB image and depth map with a resolution of 1280×720 compressed in JPEG format, and an uncompressed mesh composed of 200000 vertices (encoded as three 32-bit floating point coordinates per vertex), this would require approximately a bandwidth of 1 Gbps. This corresponds to the requirements of the Magic Leap™ AR Cloud platform [62] (mentioning between 480 Mbps and 1 Gbps).

To optimize bandwidth usage and reduce its processing load, an XR device may request a subset of the 3D model of the surrounding space from the server. The desired subset may be represented as a bounding volume that indicates the spatial region-of-interest for which the XR device wishes to receive spatial mapping data. For example, this functionality is enabled in OpenXR™ using the XR_MSFT_scene_understanding extension. An application may pass one or more bounding volumes when calling xrComputeNewSceneMSFT. These bounding volumes are used to determine which scene components to include in the resulting scene. Scene components that intersect one or more of the bounding volumes are included, and all other scene components are excluded. The bounding volumes can be indicated in the form of an oriented box, sphere, or frustum.

In addition to a spatial region-of-interest, applications may also indicate a desired Level-of-Detail (LoD) for the mesh, specifying the desired number of triangles per volume. The OpenXR™ extension XR_MSFT_scene_understanding specifies four possible levels: coarse, medium, fine, and unlimited.

#### 4.1.5.2		Example: Microsoft™ HoloLens™

HoloLens™ [9] has built-in cameras that continuously scan the environment, allowing it to construct virtual world geometry for real-world objects. Figure 4.1.5.2-1 demonstrates examples of 3D models constructed by HoloLens™.

Figure 4.1.5.2-1: HoloLens™ 3D Model construction and spatial mapping (Source: https://docs.unity3d.com/cn/2018.3/Manual/SpatialMapping.html, Unity™)

In HoloLens™, spatial surfaces are represented by dense triangle meshes. To minimize the required processing and storage resources, applications can enable a mesh caching scheme appropriate for their needs. This scheme determines which meshes to keep, which to discard, and when to update the mesh for each spatial surface.

An application can define the regions of space in which the application wishes to receive spatial mapping data by defining bounding volumes. It is recommended in the documentation that new spatial surfaces and updated spatial surfaces near and in front of the user are given higher priority by the application when spatial data is requested.

HoloLens also enables visualization of the spatial mesh to give the user an understanding of the detected spatial surfaces. An application can use this feature, for example, to help the user when the user is trying to place a virtual object onto a surface.

More details on spatial mapping in HoloLens™ is provided in [9].

#### 4.1.5.3		Example: Qualcomm™ Snapdragon™ Spaces

Spatial meshing enables real-time creation of a 3D map of the surrounding environment, allowing applications to calculate spatial relationships, e.g., to avoid obstacles, and achieve realistic occlusions. Developers can specify mesh quality and update intervals for efficient processing.

// Initialize meshing MeshManager meshManager = new MeshManager(); meshManager.StartMeshing(MeshQuality.High, updateInterval: 1.0f); // Retrieve mesh data in update loop foreach (var mesh in meshManager.CurrentMeshes) { Vector3[] vertices = mesh.Vertices; int[] indices = mesh.Indices; Debug.Log($"Mesh - Vertices: {vertices.Length}, Indices: {indices.Length}"); }

In this code snippet, StartMeshing initiates the meshing process with specified quality and update intervals. The CurrentMeshes property provides access to the latest mesh data, including arrays of Vertices and Indices for each mesh segment, which can be used for rendering or spatial analysis in the application.

#### 4.1.5.4	Example: Magic Leap™ AR Cloud

AR Cloud [62] is a connected service that enables Magic Leap 2™ devices to access extensive spatial data. It allows digital twin administrators to create Shared Spaces and manage device-side scans through a web-based console. A key feature of AR Cloud is its ability to merge scans from different users into a unified 3D model reconstruction accessible from each client. The service can run on the cloud on providers such as AWS™, GCP™, and Azure™.

### 4.1.6	Segmentation and labelling

#### 4.1.6.1		Introduction

3D Semantic Segmentation is a task that involves dividing a 3D point cloud or a 3D mesh into semantically meaningful parts or regions. The goal of 3D semantic segmentation is to identify and label different objects and parts within a 3D scene. This is similar to semantic image segmentation where sections of an image are separated into clusters of pixels relating to corresponding objects, with respective classifications.

The three main AR SDKs (Meta™, Google™, and Apple™) support segmentation and labeling solutions. However, the domain in which these functions are performed (i.e., 2D vs. 3D) may differ from one SDK to another.

For segmentation and labeling, the following input data may be used:

-	A set of reference labels (objects that will be segmented and labeled)

-	A non-segmented 3D model (3D point cloud or 3D mesh)

The output of the segmentation and labeling function is a 3D model that is composed of a number of object models and associated labels. The segmentation function can be part of the 3D model construction function. In this case, the 3D model construction function may directly output a segmented and labeled 3D model.

If the 3D model generated by the 3D model construction function is segmented and labeled, it can be described as a hierarchical graph of objects. Some 3D scene description formats that support a hierarchical graph include: glTF [44], USD [45]

Labeling is done by attaching a label (e.g. a string value) to each 3D object. There is no defined format that specifies a list of possible labels. In some 3D scene description formats, fields carrying application-specific data may be used for labeling.

#### 4.1.6.2		Example: Meta™ Space Setup

Space Setup [10] automatically identifies and marks furniture in an indoor room. Segmented objects in Space Setup may assigned one of the following labels: floor (not displayed), ceiling (not displayed), wall (not displayed), door, table, sofa, storage, screen, or bed. Objects are represented with bounding boxes around each identified object, and the segmentation can be adjusted manually.

Figure 4.1.6.2-1: Space Setup (Source: https://www.uploadvr.com/quest-v64-undocumented-features-furniture-recognition-multimodal/)

#### 4.1.6.3		Example: Apple™ RoomPlan API

RoomPlan [11] is a Swift API that utilizes the camera and LiDAR Scanner on iPhone and iPad devices to create a 3D floor plan of a room, including key characteristics such as dimensions and types of furniture. The 3D object-detection pipeline recognizes 16 object categories directly in 3D.

Segmented objects in the RoomPlan API may be assigned one of the following labels: storage, sofa, table, chair, bed, refrigerator, oven, stove, dishwasher, washer/dryer, fireplace, sink, bathtub, toilet, stairs, or TV. Objects are represented with bounding boxes.

Figure 4.1.6.3-1: RoomPlan API (Source: https://developer.apple.com/augmented-reality/roomplan/)

#### 4.1.6.4		Example: Google™ Scene Semantics API

The Scene Semantics API [12] provides real-time outdoor semantic information, which complements existing geometric information in ARCore™. Given an image of an outdoor scene, the API returns a label for each pixel across a set of useful semantic classes. ARCore™ provides a classification with 12 labels, as shown in Figure 4.1.6.4-1.

Figure 4.1.6.4-1: Scene Semantics API in ARCore™ (Source: https://developers.google.com/ar/develop/scene-semantics, Google™)

### 4.1.7	Collider generation

Colliders are used to handle collision between objects. A collider is a component that defines the shape of an object for the purposes of physical collisions [13]. Colliders are invisible, and do not need to be the same shape as the object mesh.

The consistent handling of collisions between virtual and real objects requires real objects colliders. Hence, one of the functions provided by spatial computing platforms is the creation of colliders used by physics simulation engines. These colliders are generated based on the input 3D model of the surrounding environment.

Collision shape of the object may be defined using either primitive shapes or mesh-based shapes.

Primitive shapes, such as a box, sphere, capsule, cylinder, or cone, are best in terms of memory and performance but do not necessarily reflect the actual shape of the object. They are calculated based on the object’s bounding box [14].

Mesh-based shapes (e.g., convex hull or mesh) are calculated based on the geometry of the object and are therefore a better representation of the object.

Figure 4.1.7-1: Combination of primitive colliders [15]

Figure 4.1.7-2: Mesh collider [16]

Input data can be:

-	The 3D Model

The output of the collider generation function is a set of colliders, possibly as part of the description of the 3D Model.

In the case of primitive shapes, the MPEG_scene_interactivity extention of ISO/IEC 23090-14 (MPEG-I Scene Description) [47] and the KHR_collision_shapes glTF extension support simple geometric collider shapes such as box, capsule, cylinder, and sphere.

In the case of mesh colliders, the object’s mesh and collider’s mesh may be contained in the same descriptive file if the format of the file supports more than one mesh per node (e.g., MPEG-I Scene Description or USD). Alternatively, the collider mesh shape associated with an object may be described separately using a simple format (e.g., OBJ [40], STL [43], or FBX [42]).

### 4.1.8	Light extraction

The consistent rendering of virtual and real objects, including occlusion and lighting/shadowing aspects, requires a 3D model and the detection/extraction of light parameters. The 3D model (from a certain point of view) can be used by the rendering engine for compositing.

Lighting estimation is a technique used by augmented reality platforms to match the lighting of the virtual objects in the scene to the lighting of the real world surrounding the viewer. Real light data provide different parameters that represent different characteristics of a light source. Those parameters can be used to instantiate a virtual light (i.e., a virtual representation of a real light) with the same properties, to achieve consistency between real and virtual scene. This virtual light is set with the same pose as the real one.

In ARCore™, the Lighting Estimation API provides detailed information about the lighting in a scene [17].

Figure 4.1.8-1: Rendering without light extraction (left) vs. with light extraction (right), Real object shadow highlighted in red and virtual object shadows in green (Source: https://developers.google.com/ar/develop/lighting-estimation, Google™)

Figure 4.1.8-1 shows an example of an inconsistency between the shadows without light extraction (left) and a more consistent rendering with light extraction.

Some algorithms, based on cast shadow [18], use the 3D model of the real scene to estimate light source.

For light estimation, the following input data can be used:

-	Sensor data:

-	Images captured by AR Device

-	Poses of AR Device

In XR applications, different types of light, each with specific properties, are possible and may include:

- Point light: is a light that is located at a point in the scene and emits light in all directions equally. Point lights have a specific range and only affects objects within this range.

- Spot light: is a light that is located at a point in the scene and emits light in a cone shape. It emits light in one direction and is constrained to a specific angle, resulting in a cone-shaped region of light.

- Directional light: is a light that is located infinitely far away and emits light in one direction only.

- Area light: a light that is defined by a rectangle or disc in the scene, and emits light in all directions uniformly across its surface area but only from one side of the rectangle or disc.

- Environmental light: is composed of image(s) of the surrounding environment including light sources. Several formats may be used for the image(s):

  - Cube Map: The environment is projected onto the faces of a cube and stored as six square images.

  - Equirectangular: image resulting from the projection of the surface of a spherical environment map.

Formats developed for virtual lights can be used to transport the characteristics of real lights.

### 4.1.9	Semantic perception

The semantic perception refers to the use of artificial intelligence and machine learning (AI/ML) techniques to interpret and understand the real-world environment captured by AR devices. This function enables the identification, classification, and contextual understanding of objects, surfaces, and scenes in both 2D and 3D spaces.

The semantic perception builds upon the outputs of other spatial computing functions such as segmentation and labelling (as defined in clause 4.2.6). It enhances the XR experience by enabling applications to reason about the environment, support intelligent interactions, and adapt content based on the recognized context. It may be performed locally on the AR device or remotely on edge/cloud servers, depending on the computational complexity and latency requirements. The output of this function may be integrated into the XR Spatial Description and used by other spatial computing functions or the scene manager.

For semantic perception, the following input data may be used:

-	Sensor data:

-	Images captured by AR device

-	Depth maps

-	Pose of AR device

-	3D models (meshes or point clouds)

-	Predefined label sets or ontologies (optional)

The output of the semantic perception function includes:

- 	a 3D model that is composed of object models and associated labels (similar to the output data of the Segmentation and Labelling function defined in 4.1.6)

- 	Semantic representation (representation of objects and their semantic or spatial/temporal relationships (e.g., adjacency, containment, etc.)

### 4.1.10	Real object removal

The removal of real objects in an XR application is also known as diminished reality (DR) which removes reality components from the viewer's perspective. Unwanted elements are recognized and replaced by other image elements, creating a plausible and consistent overall impression for the viewer.

Diminished reality has the potential to address a challenge in AR where virtual objects cannot be placed due to real-world obstacles. The IKEA™ Kreativ app, Figure 4.1.10-1, is an example of diminished reality.

Figure 4.1.10-1: Example of object removal in IKEA™ Kreativ (Source: https://www.ikea.com/gb/en/newsroom/stories/ikea-kreativ-launches-making-home-design-even-easier-and-more-accessible-for-the-many-pub71d09380/, IKEA™)

To remove a real object from the viewer’s perspective, the object needs to be hidden and replaced by what is behind it (background image).

NOTE: 	Due to hardware and latency constraints of optical and camera passthrough devices, diminished reality is realized through overlays, which may be prone to distortion with slight pose changes.

To hide real objects in a scene, several tasks must be performed:

-	Capture the scene.

-	Segment the scene (2D or 3D).

-	Select objects to hide.

-	Apply masks to selected objects.

-	Fill the masked areas with an inpainting process.

Using deep learning, it is possible to identify and select objects in an image. The segmented 3D model of a real scene can also be used to select target objects. A mask is then positioned over the selected areas.

To hide selected objects, the masked areas are filled with background information. This process is known as inpainting.

In-painting diminished reality is a technique in which parts of a scene are reconstructed or "painted over" using information in the source image about textures and colors around the object or the area to be reduced. There are two common methods for removing targets from videos or images. The first method fills in the missing regions by using readily available background images or essential information captured beforehand. The second method uses the information around the target or the similarity of textures to artificially produce the filling content (usually using some deep learning technique), an approach also known as image inpainting.

To hide real objects in a scene, the following input data can be used:

-	Sensor data

-	Images captured by AR Device

-	pose of AR Device

-	Depth map (image or texture)

-	Mesh captured by AR Device

When rendering the DR scene, two methods can be applied to hide objects and the shadows casted by the object to be removed:

-	A set of images containing transparent areas and non-transparent areas is used. Non-transparent areas hide real objects and their shadows.

- 	3D objects (meshes and textures) are used to hide reals objects and their shadows.

Image with transparency and 3D objects are possible outputs of the “Real Object Removal” function.

Hiding an object does not erase its shadow. To improve the XR experience, shadow removal could also be done. The aim of shadow removal is to restore the lighting conditions, color, and texture in the shadow regions. Recent shadow removal techniques predominantly adopt deep learning-based approaches.

Shadow removal methods typically involve two key steps: shadow detection and shadow removal, based on the detected shadow mask. Prior-based shadow removal methods are generally based on physical models and adopt the prior information such as gradient, illumination, and regions to remove shadows. The emergence of deep learning has greatly improved the performance of shadow removal. In all recent publications, deep learning is used to remove shadows.

To remove a shadow, several tasks must be performed:

-	Capture the scene

-	Detect shadows (associated to hidden objects)

-	Apply masks to detected areas

-	Fill the masked area with an inpainting process

When rendering the DR scene, the same methods used to hide objects can also be applied.

Sensor data and output are the same as before.

### 4.1.11	Summary of spatial description formats

This section describes the common output data formats for the spatial computing functions defined in clause 4.2. Table 4.1.11-1 provides a list of the output data for each spatial computing function and the corresponding format.

Table 4.1.11-1: Output data of spatial computing functions.

| Function | Data | Description | Format Example | Possible Representation |
| --- | --- | --- | --- | --- |
| World Tracking | Feature Map | Features to compute a pose in XR Space. | Array of features |  |
|  | Feature | 3D point associated with a descriptor, the descriptor format depends on the feature type. | SIFT (Scale Invariant Feature Transform) | Typically, a vector of 128 floats. |
|  |  |  | ORB – Oriented FAST Rotated BRIEF (Binary descriptor) | Typically, a 32-bit descriptor. |
|  |  |  | BRIEF – Binary Robust Independent Elementary Features) | Typically, 32-bit descriptor. |
|  |  |  | FREAK – Fast Retina Keypoint | Typically, 64-bit descriptor. |
|  |  |  | SURF – Speeded-Up Robust Features | Typically, vector of 64-bit or 128-bit floats. |
| Relocalization | Pose | Pose of the AR device in XR Space | Position: 3D vector Orientation: quaternion | Position: 3 floating point values. Oriantation: 4 floating point values |
|  | XR_space_id | An identifier for the XR Space used as a reference. | String |  |
| Anchoring | Id | An identifier of the new trackable (when the anchoring function is invoked by a producer). | String |  |
|  | Pose | Pose of the anchor in relation to the device according to a trackable (when the anchoring function is invoked by a consumer). | Position: 3D vector Orientation: quaternion | Position: 3 floating point values. Oriantation: 4 floating point values |
| 3D Model reconstruction | Model | 3D model(s) of surrounding space. | Non-segmented model (e.g., point cloud, mesh with/without attributes) | OBJ, STL, PLY [41], FBX, glTF |
|  |  |  | Segmented model | glTF, USD |
|  | Bounding volume | Spatial region-of-interest indicating desired subset of the 3D model. | Oriented box, sphere, frustum. |  |
|  | Level-of-Detail | Desired level-of-detail for the 3D model (e.g. number of triangles per volume). | Enum: coarse, medium, fine. |  |
| Semantic Perception | Objects | A list of 3D objects resulting from the segmentation and labeling function. | Array or a hierarchical node graph. |  |
|  | Object | A 3D object in the captured world. |  |  |
|  | Mesh | Segmented object. | Mesh |  |
|  | Label | Label of segmented object. | String |  |
|  | Semantic representation | Representation of objects with semantic relationships (e.g., adjacency, containment) | Graph of objects (Nodes represent objects; edges represent spatial or semantic relationship as textual description) | Graph, List, Array of textual description |
| Collider Generation | Colliders | Set of colliders (not combined with associated objects). | Array of colliders |  |
|  | Collider | A collider object. | Primitive (e.g., sphere with radius) |  |
|  |  |  | Mesh | OBJ, STL, FBX |
|  | Colliders | Part of description of the 3D Model (when combined with associated objects). | Hierarchical node graph | MPEG-I SD, USD [46] |
| Segmentation and Labeling | Objects | A list of 3D objects resulting from the segmentation and labeling function. | Array or a hierarchical node graph. |  |
|  | Object | A 3D object in the captured world. |  |  |
|  | Mesh | Segmented object. | Mesh |  |
|  | Label | Label of segmented object. | String |  |
| Light Extraction | Lights | Set of extracted lights | Array |  |
|  | Light | A description of a light source that includes a set of parameters that depend on the type of the light. Possible types include: point, directional, area, spot, texture-based, or image-based light. | Object | For point light, the parameters include: pose, intensity, color, and range. |
| Real Object Removal | Objects | A set of textured 3D objects. | Mesh + texture | OBJ, STL, PLY, FBX, glTF |
|  | Images | Images with transparency. |  | PNG, TIFF, TGA, BMP |

## 4.2	Quality of experience (QoE) for spatial computing

TR 26.812 [28] addresses QoE metrics aspects when XR devices are based on the architecture defined in TS 26.119 [25]. Among other things, the document:

- identifies relevant XR QoE Metrics and their impacts on the user experience, and

- defines the measurement and derivation of relevant XR QoE metrics in the device architecture based on TS 26.119.

The AR QoE reference model and observation points, defined in TS 26.119 and documented in TR 26.812, are illustrated in Figure 4.2-1.

Figure 4.2-1: AR/MR QoE reference model and Metrics Observation Points [28].

The following QoE metrics from TR 26.812 are relevant to spatial computing functions and services:

-	Trackable creation delay corresponds to the delay between the time of the trackable creation request and the time when the related XR space (i.e., the frame of reference in which the 3D coordinates are expressed) is created.

- 	Trackable detection to render to photon corresponds to the delay between the time of the trackable pose request leading to the detection of the trackable and the time when the virtual content is displayed in the user’s environment.

- 	Tracking pose prediction error mainly refers to the relative pose error which indicates the deviation of the relative pose in the real world and the predicted pose. This can be observed at OP-1 and derived by comparing the predicted spaces’ locations and real space locations.

- 	Trackable untracked ratio corresponds to the ratio between the number of frames where the trackable is not tracked and the total number of frames during the observation period in which no detection process is launched.

Table 4.2-1 lists the QoE metrics that relate to some of the spatial computing functions documented in clause 4.1.

Table 4.2-1: Spatial computing functions and relevant QoE metrics.

| Spatial computing function | QoE Metrics |
| --- | --- |
| Anchoring | - Trackable creation delay - Trackable detection to render to photon delay - Tracking pose prediction error - Trackable untracked ratio |
| Relocalization | - Tracking pose prediction error |

# 5	Related standardization work

## 5.1	3GPP

The Feasibility Study on Localized Mobile Metaverse Services (TR 22.856) [35] investigates specific use cases and service requirements for 5GS support of enhanced XR-based services as well as potentially other functionality, to offer shared and interactive user experience of local content and services, accessed either by users in the proximity or remotely. The technical report documents several use cases which require the handling of XR Spatial Descriptions, including: a localized mobile metaverse service enabler, a spatial anchor enabler, and a spatial mapping and localization service enabler.

3GPP has also conducted studies on support for XR services (TR 26.928) [2] in general and AR services (TR 26.998) [3] in particular with documented use cases where he knowledge of the real world is essential for the localization of the AR device and for a seamless insertion of virtual content into the user’s real environment. The potential work identified by TR 26.998 includes specifying support for AR relevant functionalities such split-rendering or spatial computing on top of a 5G System based on a generic architecture for real-time media delivery.

In Release 18, support for AR anchoring has been defined in TS 26.119 [25] and TS 26.143 [26] based on the MPEG_anchor glTF extension. A visualization space format defining a volume free of obstacles to insert virtual content into the user’s environment is defined in TS 26.119 [25] and TS 26.264 [27]. Quality-of-Experience (QoE) metrics related to AR anchoring are defined in the TR 26.812 [28].

3GPP has also been working on a study on an application enablement architecture for mobile metaverse services. The technical report of this study, TR 23.700-21 [29], documents a spatial mapping service enabler which includes procedures to produce, update, and delete spatial maps, to subscribe to event related to spatial maps, to get localization service, to register spatial map service provided by 3rd party application server, and to augment VAL UEs into spatial maps. A detailed definition of a service enabler based on the outcome of the study is defined in TS 23.437 [66].

## 5.2	Non-3GPP

### 5.2.1	Khronos

OpenXR [20] is an open standard developed by the Khronos Group that provides a set of APIs for developing extended reality (XR) applications that can run on a wide range of XR devices. OpenXR includes various vendor extensions to address various specific features related to spatial computing:

- Anchoring features: XR_MSFT_scene_marker, XR_VARJO_marker_tracking, and XR_ML_marker_understanding extensions are used to track 2D images in the real world (e.g., QR code or other vendor markers). XR_HTC_anchor, XR_MSFT_spatial_anchor extensions are used to create and track spatial anchors i.e. a point in the real environment.

- 3D construction features: XR_MSFT_scene_understanding, XR_FB_scene_capture, and XR_FB_triangle_mesh extensions are used to retrieve representations of the real environment.

### 5.2.2	ETSI

The ETSI Industry Specification Group AR Framework (ISG ARF [21]) has developed a framework for AR components and systems. This framework introduces the characteristics of an AR system and describes the functional building blocks of the AR reference architecture and their mutual relationships.

In that architecture, the World Storage block delivers information to other functions and subfunctions concerning the representation of the real world (e.g., information for relocalization, 3D objects recognition and identification, and AR authoring). Additionally, it receives information from other functions (e.g., 3D Mapping and 3D Object Recognition) to update the representation of the world. It can also convert the world representation (Scene Meshing) or extracts part of this representation (Object 3D Segmentation).

In that context, the concept of World Graph was introduced in ETSI GS ARF 004-2 [22]. A World Graph describes a scene of the real world used at runtime by an AR system to spatially register AR assets with the real world. It consists of trackables and world anchors spatially linked together. Trackables denote an element of the real word that can be tracked. Several types of trackables are specified, e.g. FIDUCIAL_MARKER, IMAGE_MARKER, MAP, GEOPOSE or audio and UWB (Ultra-WideBand) trackables. A World Anchor represents a pose in the real world in relation to one or multiple Trackables. APIs for the creation and management of this world representation are presented in ETSI GS ARF 005 [23].

Figure 5.2.2-1: Example of World Graph as detailed in ETSI ARF

The World Analysis block uses the spatial description contained in the World Graph and combines it with the data coming from the AR runtime, such as video capture and other sensor data, to provide AR services like the localization of the user device, the recognition and the identification of elements present in the real world, the 3D reconstruction of the real environment and the tracking of real object. The specifications of the communication between the World Analysis, the World Storage, and Scene Management blocks is described in ETSI GS ARF 004-4 [32].

### 5.2.3	MPEG

The MPEG-I Part 14 Scene Description (ISO/IEC 23090-14) [24] standard specifies how to reference and position 2D and 3D assets within a scene, enabling proper rendering and creating rich 3D scenes with real-time media delivery and interactivity. It defines several extensions to the Khronos glTF 2.0 specifications, related to the inclusion of dynamic and temporal media, interactivity and augmented reality.

In this context, the MPEG_anchor extension enables a content creator to describe the spatial relationships between the virtual objects and particular real-world locations based on the definition of AR anchor and trackable objects. To support a variety of indoor and outdoor AR experiences, several types of trackables are defined, such as controller-based, floor, viewer, horizontal, or vertical planes, 2D or 3D marker, geospatial coordinates or application specific.

The MPEG_anchor extension consists of Trackable and Anchor objects. In an amendment to the 2nd edition of ISO/IEC 23090-14 (WG03N01454), a recommendedSpatialComputingConfig object is added to the Anchor object defined in the MPEG_anchor extension. This new object provides a set of recommended parameters which could be used for the configuration of the XR spatial computing functions. The object contains the following parameters in Table 5.2.3-1.

Table 5.2.3-1: Parameters of the recommendedSpatialComputingConfig object.

| Parameter | Function |
| --- | --- |
| scanOptions | Array of options (enumeration) for the scan computation (PLANE, PLANAR_MESH, VISUAL_MESH, COLLIDER_MESH, FREE_VOLUME, BOUNDING_BOX, TEXTURED_MESH). |
| scanDetails | Specifies the required level of detail for the mesh (quantity of the geometric primitives and texture resolution of textured mesh). |
| scanUpdate | Specifies the frequency at which the spatial description must be updated. |
| scanVolumes | Array of bounding volumes that determine the spaces where scanned objects must be provided. Real scanned objects that intersect one or more of the bounding volumes should be provided, and all other objects ignored. Possible types of bounding volumes are SPHERE, BOX and FRUSTUM. |
| realSemantic | Semantic descriptions of nodes that are needed (“table”, “room”, “chair”, “wall”, “light”, “freespace”, etc.). |
| lightOptions | Array of options (enumeration) for light extraction (DIRECTIONAL_LIGHT, ENV_LIGHT, POINT_LIGHT, SPOT_LIGHT, AREA_LIGHT). |
| lightUpdate | Specifies the frequency at which the extraction of real light must be updated. |

When processing the MPEG_anchor extension, if a recommendedSpatialComputingConfig object is present, the Presentation Engine checks if it can retrieve the recommended spatial description, specified in the parameters in recommendedSpatialComputingConfig. If all the recommended parameters are not satisfied, the rendering can continue with the available spatial description, but possibly with a degraded XR experience.

As part of its glTF 2.0 extensions, Khronos has defined a set of static light sources for use with glTF 2.0 scenes. These extensions include the KHR_lights_punctual, EXT_lights_ies, and EXT_lights_image_based extensions. As part of its second edition, ISO/IEC 23090-14 expand these light extensions to describe dynamic lighting of a scene. It includes the MPEG_lights_texture_based extension (for Image Based Light) and the MPEG_lights_punctual extension (for Punctual light) [24].

### 5.2.4	Open Geospatial Consortium (OGC)

OGC [30] is an organization dedicated to developing Standards for improving access to geospatial, or location information. OGC’s free and open geospatial Standards define interoperable approaches to encode, access, process or visualize real environment data. Some of these standards are related to localization services (GeoPose) or description language for real world representation (e.g., 3D Tiles, CityGML, and IndoorGML).

The GeoPose standard [33] provides a framework for representing the position and orientation of objects in 3D space that can be used by AR applications. It allows for accurate spatial referencing of physical objects and digital content in relation to the physical world. GeoPose combines geographic coordinates with orientation data to create a comprehensive spatial reference.

OGC is also working on the POI (Point of Interest) Conceptual Model [34] standard. A POI is a location about which information of general interest is available A Point of Interest (POI) can range from a basic set of coordinates with an identifier to a more complex representation, such as a 3D model of a building that includes names in multiple languages, details about operating hours, and a civic address.

### 5.2.5	Open AR Cloud

Open AR Cloud [31] is an initiative working on the development of open and interoperable AR Cloud technology, data and standards to connect the physical and digital worlds. Open AR Cloud has developed the Open Spatial Computing Platform (OSCP) which includes a set of standard components that enables the authoring, publishing, discovery, and consumption of digital content such as XR experiences that has been anchored to the real world. OSCP is based on 3 core functions:

- GeoPose: Developed in the Open Geospatial Consortium (OGC), to obtain, save, record, share, and communicate the geospatial position and orientation of any real or virtual object with 6 degrees of freedom. The OSCP GeoPoseProtocol API uses a request/response protocol to localize an equipment from a set of sensor data.

- Reality Modelling Language: Provides a standardized encoding of geometry, semantics, properties, and relationships (linked data) of a dynamic reality.

Spatial discovery services: Provide seamless access to content, services, applications communication channels that are available to the user at their location.

# 6	Mapping of spatial computing to 5G services

## 6.1	Introduction

Using the Reference Architecture for Media Delivery defined in clause 4.1.2.2 of TS 26.506 [65] as a reference architecture, it is also possible to directly map specific spatial computing functions into the generalized functions in order to support spatial computing services. Spatial Computing Functions are located in the Media AS and in the XR Runtime. These functions deliver XR Spatial Description data.

## 6.2	Terminal Architecture

The terminal architectural breakdown is based on the XR baseline terminal architecture in clause 5.1 of TS 26.119 [25]. The figure depicting the XR baseline terminal architecture is replicated here as Figure 6.2-1 for convenience.

NOTE: 	The term “XR baseline client” from TS 26.119 [25] is changed here to “XR baseline terminal”, as the provided architecture also includes other entities (e.g. XR Runtime, Scene Manager) in addition to the Media Client (Media Session Handler and Media Access Function).

Figure 6.2-1 - XR baseline terminal architecture.

The XR baseline terminal consists of the following components:

- The Media Access Function is responsible for the delivery of any metadata or local media to the spatial computing functions in the AS.

- The Scene Manager is responsible for the parsing of the description provided by the AS as output from the spatial computing functions (XR Spatial Description). It is also responsible for setting up and managing the XR session with the XR Runtime.

- The Presentation Engine is responsible for rendering the scene components based on the input from the Scene Manager.

- The XR Source Management is responsible for gathering timed metadata such as sensor data and pre-processed sensor information and making it available to the XR Application or the Media Access Function for sending over the uplink..

- The XR Runtime is a set of functions provided by the XR Device to the XR Application to support XR experiences. It may include some spatial computing functions that can be performed locally on the device.

- The XR Application is a software application that runs on an XR device and offers an XR experience based on an XR Runtime. An XR Application may invoke the spatial computing functions included in the XR Runtime.

## 6.3	General Architecture for Spatial Computing

Figure 6.3-1 – Spatial computing architecture.

The architecture includes the following network functions and UE entities:

- Media Application Provider: Offers the spatial computing service and provisions support for spatial computing sessions offered by it.

- Media Application Function (Media AF): This function is responsible for QoS allocation in the 5G Core and providing service configuration information to the Media AS and Media Session Handler.

- Media Application Server (Media AS): This function is responsible for establishing the spatial computing session with the Media Client and monitoring the server’s edge resource usage. It hosts a Spatial Computing Function which can manage and run the spatial computing functions.

- Media-aware Application: The application running on the UE that makes use of 3GPP-defined APIs to invoke the Media Session Handler and/or the Media Access Function.

- Media Client: A UE internal function comprising a Media Session Handler and a Media Access Function.as well as other functional blocks such as XR Source Management, Scene Manager and Presentation Engine.

- Spatial Computing Client (SCC): This function is responsible for discovering the UE spatial computing capabilities and negotiating with the Media AS to agree on the spatial computing session.

NOTE: The SCC is a subset of the Media Client and includes a Media Access Function and a Media Session Handler. Whether the SCC would also include the Scene Manager and XR Source Management is FFS.

- Media Session Handler (MSH): This entity on the UE is responsible for the control plane signalling with the Media AF to establish a spatial computing session.

- Media Access Function: This function is as defined in TS 26.506 with the capabilities to send sensor data and receive XR Spatial Descriptions from the Media AS.

- Scene Manager: a set of functions that supports the application in arranging the logical and spatial representation of a multisensorial scene based on support from the XR Runtime. The Scene Manager composes the scene using the XR Spatial Description data.

- XR Source Management: management of data sources provided through the XR Runtime. It retrieves the sensor data from the XR Runtime and provides them to the Media Access Function.

- XR Runtime: Set of functions provided by the XR Device to the Media-aware Application to create XR experiences. It may include some spatial computing functions.

The relevant interfaces shown as solid lines in Figure 6.3-1 are:

- M1: The Media Application Provider provisions the spatial computing service through M1.

- M4: The signaling as well as the data delivery between Media Access Function and Media AS is through M4.

- M5: The MSH and the Media AF may exchange spatial computing configuration related information through the M5 interface.

- M6: Reference point between the Media-aware Application and the MSH for the purpose of configuring the Media Session Handler.

- M7: The Spatial Computing Client discovers the UE spatial computing capabilities through the M7 interface.

- M11: This interface may be used to convey QoS allocation and QoE information related to spatial computing functions between the MSH and the Media Access Function.

## 6.4 	Call flow for spatial computing session setup and operation

The spatial computing operation can be described by the call flow in Figure 6.4-1.

Figure 6.4-1 - High-level call flow for spatial computing session setup and operation.

The steps are:

0.	The Scene Manager acquires the scene description and discovers the needed spatial computing functions for the XR experience.

1.	The Media-aware Application initiates a session by configuring the Media Access Function at reference point M7. The MAF informs the Media Session Handler about the session and retrieves relevant policy and configuration information.

2.	The Media Access Function informs the Media Session Handler about the session and retrieves the relevant policy and configuration information at reference point M11.

3.	The Media Session Handler communicates with the Media AF for the AS discovery (in the case of edge computing) and the configuration of quality-of-service parameters at reference point M5.

4.	The Media Session Handler passes the information configured in step 3 to the Media Access Function via the interfaces M6/M11.

5.	The Media Access Function in the SCC establishes the session with the Media AS at reference point M4.

6. 	The Media-aware Application configures the XR Runtime. This configuration enables the XR Runtime to provide sensor and control data needed for the spatial computing functions.

NOTE: Sensor data may include, for example, the pose of the AR device. Control data may include, for example, coordinates of a bounding volume to request from the Media AS as a spatial region of interest, i.e., a subset of 3D world model.

7.	The Source Manager retrieves sensor and control data from the XR Runtime and provides them to the Media Access Function.

8.	The Media Access Function sends sensor and control data to the Media AS.

9.	The Media AS uses that data to invoke the spatial computing functions and compute the XR Spatial Description.

10.	The Media AS transmits the generated XR Spatial Description to the Media Access Function in the Spatial Computing Client.

11.	The Spatial Computing Client may complete XR Spatial Description using local sensor data.

12.	Spatial Computing Client provides the XR Spatial Description to the Scene Manager.

13.	The Scene Manager composes the scene using the XR Spatial Description.

14. The Scene Manager provides the composed scene to the Presentation Engine.

15. The Presentation Engine renders the composed scene based on the input from the Scene Manager.

## 6.5	Cross operation with other architectures and media service enablers

### 6.5.1	Extended Spatial Computing architecture for Edge Computing

#### 6.5.1.1	General

The spatial computing general architecture can be extended to add support for media processing in the edge. The extended architecture is an integration of the spatial computing general architecture defined in the present document with the architecture for enabling Edge Applications defined in TS 23.558 [50] and TS 26.501 [51].

The extended spatial computing general architecture is required to support both Media client-driven as well as Media Application Function (AF)-driven management of the edge processing session.

The Media Application Provider may request the deployment of edge resources as part of the Provisioning Session.

-	The Media Application Provider provisions edge resources through reference point M1, in a similar fashion as defined in TS 26.501 [51], enabling Media client-driven and/or Media AF-driven edge configuration.

-	In the Media client-driven approach, the Media-aware Application becomes aware of the support of edge processing in the network and takes steps, such as using the APIs at reference point EDGE-5, to discover and locate a suitable Media AS instance in the Edge DN, similar to the process defined in clause 8.1 of [51].

-	In the Media AF-driven approach, the Media Application Provider provisions the Media AF to deploy edge processing for the media sessions of the corresponding Provisioning Session, similar to the process defined in clause 8.2 of [51]. In this case, instantiating edge resources is the responsibility of the Media AF, based on monitoring the load of the deployed EAS instances. A Media-aware Application may become aware of the deployed EAS through the Media Application Provider from the MSH through reference point M5 (and possibly M6). This appropriate EAS instance for a particular EAS Client to use is discovered by means of DNS resolution with support from the DNS server (e.g., EASDF/DNS resolver) as defined in TS 23.548 [52].

When the Media-aware Application is a web application, the implementation of the interface at reference point EDGE-5 to discover the Media AS/EAS location by accessing the EEC is difficult if the Web browser does not implement interfaces necessary for supporting edge-enabled media-aware applications/services.

Figure 6.5.1.1-1: Edge-enabled spatial computing architecture.

#### 6.5.1.2	Edge Application Server (EAS)

EAS is the application server resident in the Edge Data Network (EDN), performing edge-based processing for AR functionalities such as split rendering and spatial computing. The Application Client (AC) connects to the EAS in order to avail the services of the application with the benefits of Edge Computing. The functions of Edge enabler Client (EEC), Edge Enabler Server (EES), Edge Configuration Server (ECS) are as defined in TS 23.558 [50].

It is possible that the server functions of an application are available only as an EAS. It is also possible that certain server functions are available both at the edge and in the cloud as an EAS and an Application Server resident in the cloud.

The EAS can use the 3GPP Core Network capabilities in the following ways, all of which are optional to support:

a)	invoking 3GPP Core Network capabilities via the edge enabler layer through the Edge Enabler Server (EES)

b)	invoking 3GPP Core Network function (e.g., PCF) APIs directly, if it is an entity trusted by the 3GPP Core Network; and

c)	invoking the 3GPP Core Network capabilities through the capability exposure functions, i.e., SCEF/NEF/SCEF+NEF.

#### 6.5.1.3	Edge Interfaces

Based on the extended architecture, the following interfaces are defined for performing edge-based processing for AR functionalities such as split rendering and spatial computing:

1.	A Media AF that is edge-enabled shall support EES functionality including:

-	EDGE-1 API for supporting registration and provisioning of EEC functions, and discovery by them of EAS instances.

-	EDGE-3 API towards the EAS function of Media AS instances.

-	EDGE-6 API for registering with an ECS function.

-	EDGE-9 API for media session relocation.

2.	A Media AS that is edge-enabled shall support EAS functionality including the EDGE-3 API for registration with the EES.

3.	An MSH that is edge-enabled should support EEC functionality including:

-	Invoking the EES function using the EDGE-1 API.

-	Invoking the ECS function using the EDGE-4 API.

-	EDGE-5 API exposed to the Application Client.

4.	A media-aware application that is edge-enabled shall support Application Client (AC) functionality and should invoke the ECS function using the EDGE-5 API.

### 6.5.2	Potential Prerequisites for a Spatial Computing MSE

#### 6.5.2.1	General

Pre-requisites document what is expected to be available either from the 5G System (i.e., certain functionalities of the 5G System) or from implementation (for example functions available on the device). This section provides guidance on the pre-requisites on the 5G system and device APIs in order to host and run spatial computing functions.

#### 6.5.2.2	Pre-requisites on 5G System

Pre-requisites on the 5G System include, but are not limited to:

-	The Spatial Computing Function present in the Media AS are expected to operate in the 5G system as specified in [53] that supports functionalities and procedures to configure the quality of service (QoS) and charging information and functionality, which may be used by spatial computing functions.

-	The Spatial Computing Function present in the Media AS is expected to operate within the 5G system as specified in [53], which supports functionalities and procedures to configure policy and charging control (PCC) information as specified in [55]. Spatial computing functions may benefit from these procedures.

-	The Spatial Computing Function present in the Media AS may operate within the 5G system as specified in [53], which supports the discovery of and access to edge resources. The edge functionality may be used to deploy spatial computing functions server functionality.

#### 6.5.2.3	Pre-requisites on Device APIs and Functionality

The following assumptions for the spatial computing client are made:

-	The Spatial Computing Client may have access to an XR runtime through a well-defined external API such as the OpenXR [20] or WebXR [54] APIs.

#### 6.5.2.4	Pre-requisites on Network Support

The Spatial Computing Function on the Media AS stands to benefit from the several procedures that the network offers. These include but are not limited to:

- 	Dynamic QoS and charging,

-	Edge resources,

-	Consumption and metrics reporting,

-	Configuration Information.

#### 6.5.2.5	Pre-requisites on Edge Resources

A Media Application Provider may use the procedures defined in clause 8.6 of TS 26.510 [51] to define an edge resource configuration to be used for split-rendering sessions.

### 6.5.3	Potential mapping to Generalized IMS DC Architecture

#### 6.5.3.1	General

A potential mapping of the spatial computing function into the IMS architecture is shown in Figure 6.5.3.1-1.

NOTE: There is no IMS system architecture supporting spatial compute.

Figure 6.5.3.1-1: Generalized IMS DC Architecture to support Spatial Computing.

Spatial Computing AR MTSI Client (SR-ARMTSI Client): An AR-MTSI Client, that is responsible for acquiring the UE media capabilities and interacting with the MF during the spatial computing process.

The following reference points from Figure 6.5.3.1-1 are used to enable spatial computing over IMS:

-	Mb: Reference point to enable spatial computing between UE and MF.

-	Gm: Reference point to support communication between UE and IMS, as specified in TS 23.228 [63].

-	MDC1: Reference point for transport of data channel media between data channel media function and DCSF. MF terminates the bootstrap data channel from the UE and forward HTTP traffic between UE and DCSF via MDC1, as specified in TS 23.228 [63].

-	MDC2: Reference point for transport of data channel media between data channel media function and AR Application Server, for spatial computing functions between application server to MF. MF relay traffic on A2P/P2A application data channels between the UE and the AR Application Server via MDC2, as specified in TS 23.228 [63].

The following reference points are also used to support spatial computing related procedures:

-	DC1: Reference point between the DCSF and the IMS AS, as specified in TS 23.228 [63].

-	DC2: Reference point between the IMS AS and MF, for spatial computing related data channel media resource management, as specified in TS 23.228 [63].

-	DC3: Reference point between the DCSF and NEF, as specified in TS 23.228 [63].

-	DC4: Reference point between the DCSF and AR Application Server, as specified in TS 23.228 [63].

-	N33: Reference point between NEF and AR Application server, network exposure to enable spatial computing related applications, as specified in TS 23.501 [64].

NOTE: DC5, ISC, and N5 are out of scope.

#### 6.5.3.2	Call flow for session set up and operation

The general procedure for spatial computing session establishment and operation is shown in the call flow in Figure 6.5.3.2-1.

Figure 6.5.3.2-1: General procedure for session establishment and operation in IMS DC architecture.

The steps are as follows:

0:	The UE initiates an IMS session, including bootstrap data channel establishment, with the originating Media Function.

1:	UE sends a request to IMS AS to establish an application data channel for a spatial computing session. The UE can initiate a request for spatial computing session establishment if its spatial computing capabilities cannot meet the related compute and rendering requirements. The UE calculates which operations and spatial computing functions it can perform based on its status and decides which functions to be performed in the UE and which to be run in the IMS network.

2:	The IMS AS sends DCSF the event notifications including spatial computing related information.

3:	The DCSF receives event notifications from the IMS AS and processes the session establishment request based on the information in the notification (i.e. associated spatial computing related information). The DCSF manages (if applicable) application data channel resources at the MF which meet the spatial computing request, to instruct IMS AS to terminate the media flow of the UE to the SC Function.

4 and 5:	The IMS AS receives the data channel control instructions from the DCSF and accordingly interacts with the MF via DC2.

6:	The IMS AS sends a spatial computing session establishment request to the AR AS via the DCSF. The request may include the spatial computing functions to be run in the IMS network.

7:	The AR AS sends a description of the spatial computing output to the IMS AS via the DCSF.

8:	The IMS AS sends the media resource allocation request to the Media Function, to reserve spatial computing resources for the UE.

9:	When the resources are allocated successfully, the MF returns a successful response to the IMS AS.

10:	The IMS AS returns a successful response to the UE.

11:	Successful spatial computing session is established between the UE and MF through the application data channel.

12:	The MF exposes its current capabilities and resources to the DC-AS. The information exposed may be the hardware and software stack and the resources currently available at the MF, including the supported spatial computing functions.

13:	AR AS selects resources at the MF and asks MF to reserve these resources.

14:	AR AS transfers application source data, for example scripts and scene graph, and graphical assets to the MF.

15:	UE relevant application source data is sent to the UE.

16: UE and MF negotiate the spatial computing session configuration.

17: MF configures which spatial computing functions are to be used in the session.

18: UE sends a request to invoke a spatial computing function.

19: UE may also upload any data required as input to the desired spatial computing function. This data could be for example sensor data collected by the UE.

20: The MF runs the spatial computing function and return the resulting spatial descriptor to the UE.

# 7	Conclusions and proposed next steps

## 7.1	Conclusions

Augmented reality (AR) composites virtual objects with reality. Knowledge of the real world is essential for the localization of the AR device and for a seamless insertion of virtual content into the user’s real environment. The generation of information about the real world from the processing of sensor data may be done on the UE or delegated to a server in some cases for a number of reasons (e.g., in the case of UE devices with limited computational capabilities or low battery levels, the need for a central network function in some multi-user applications, etc.).

To support such Spatial Computing services, the following aspects have been documented in this report:

-	A set of relevant Spatial Computing functions have been identified in clause 4.1 based a number of AR use cases. For each Spatial Computing function, the input sensor data and the output Spatial Description are identified. Some examples of Spatial Description formats have also been documented.

-	A number of existing Quality-of-Experience (QoE) metrics have been identified as relevant for Spatial Computing services and a mapping of these QoE metrics to the Spatial Computing functions has been documented in clause 4.2. In particular, the anchoring and re-localization functions are mapped to the relevant metrics, but no QoE metrics or delay requirements and constraints have been documented for other functions. Additional mappings and requirements may be further studied in the future.

-	The related standardization works in 3GPP and other standardization bodies and the relevant of these works to Spatial Computing services in general, and the spatial computing functions identified in this report in particular, has been studied in clause 5, leading to the identification of some gaps:

-	Some functions are not well addressed, in particular 3D model reconstruction, segmentation and labelling, light extraction, and collider generation, described in clause 4.2, as existing standardization works mainly address the world tracking (e.g., in ETSI ARF), re-localization, and anchoring functions (e.g., in ETSI ARF and TS 23.437).

-	The UE device capabilities related to Spatial Computing is not defined in TS 26.119. This can include capabilities on the supported spatial computing functions, spatial description formats, and, based on the device capabilities, the format for requests and metadata for in-network support for spatial computing functions.

-	The support of AR is not addressed in a split rendering architecture as specified in TS 26.565

-	The mapping to 5G services:

-	A spatial computing architecture is provided in clause 6.3 based on the reference architecture for Media Delivery (clause 4.1.2.2 of TS 26.506). Call flows for spatial computing session set-up and operation involving a Spatial Computing client and the remote Spatial Computing functions located in a Media Application Server are also described in clause 6.4.

-	In addition, an extension to this architecture for edge-enablement (defined in TS 23.558 and TS 26.501) and a potential mapping to the generalized IMS DC architecture are documented in clause 6.5.

-	Guidance on the pre-requisites on the 5G system and device APIs to host and run spatial computing functions is also provided in clause 6.5.2.

## 7.2	Proposed next steps

Based on the details in the report, the following next steps are envisaged for defining a spatial computing Media Service Enabler (MSE):

- Identify a selected set of spatial compute functions that may benefit from off-device processing based on existing deployments and current industry practices.

-	Add UE Spatial Computing capabilities to TS 26.119.

- For the identified set of spatial compute functions that may benefit from off-device processing:

  - Specify the signaling and negotiation of exchanged media, sensor and control data.

  - Select interoperable formats for the media, sensor and control data.

  - Specify the configuration, and delivery of the media and sensor data, potentially identifying existing compression for that data.

  - Specify procedures and APIs to access the spatial compute MSE.

In a future release, a spatial computing enabler may be developed independently, or with support from some of the work done on the 3GPP Service Enabler Architecture Layer for Verticals (SEAL) on spatial map and spatial anchor management (TS 23.437).

Annex A:
Spatial computing use cases

# A.1	Environment Aware AR Gaming

## A.1.1	Description

AR Gaming is mentioned in TR 26.928 [2] and TR 26.998 [3]. AR gaming applies virtual models to the real world and then provides an entertaining gaming experience in a world that combines virtual and real objects. People can interact with virtual objects controlled by game operations in the foreground of the real world. AR gaming can support multi-player games where several users share the same experience in the same real environment. AR games differ from traditional video games because they leverage the user's environment to create customized scenarios using spatial computing functions.

An example cited in TR 26.998 [3] is an AR golf game. Here, The AR display renders a virtual sand table of golf course and golf ball through the spatial location on the floor of her house. Alice needs to use a certain gesture to hit the golf ball and make it go into the hole.

Another example of AR gaming can be a shooting game such as Project XRay developed on HoloLens and shown in Figure A.1.1-1. In such a game, enemies can come through the wall and users can hide behind furniture and shoot at them using a gesture or a touchscreen.

Figure A.1.1-1: HoloLens™ XRay shooting game

## A.1.2 	Potential use of spatial computing functions

In such games, spatial computing functions are first used to maintain visual consistency between the real and the virtual worlds. World Tracking is used to maintain the relative position between the user and the assets of the game. Then, Light Extraction is used to correctly light the virtual object. Finally, Mesh Construction can be leveraged to manage occlusion between real and virtual elements (by rendering it as a depth mask), for example for hiding the enemies passing behind a wall or a piece of furniture.

As other users may join to create a shared experience or create a persistent experience where a user can have a break and resume the game later, the Relocalization and Anchoring spatial computing functions may be used for correctly replacing the AR content related to the real time at any time and for any user. In that case, these functions can be hosted on a distant server accessible by all users. Placing the Golf course on a table or on the floor or making the enemies appear through the walls in the shooting game requires detection of flat surfaces, which is supported by the Anchoring function, as well as Segmentation for extracting the semantics of these surfaces.

As detailed physical interactions between real and virtual objects are particularly important for AR gaming. Collision management can be used to get realistic trajectories of the golf ball with proper interactions with the walls and furniture, or for blocking the shots in the shooting game, allowing the user or the enemies to exploit the environment for hiding. A Collider Generation function can be leveraged to do so.

# A.2	AR IoT control and supervision

## A.2.1	Description

AR IoT control is mentioned in TR 26.998 [3]. AR user interfaces can be placed in situ to create natural interactions with the IoT devices in our surroundings through gesture and voice control. For example, they can be used to interact with the sensors and actuators in smart homes as illustrated in Figure A.2.1-1.

Indeed, many IoT devices are present in the home and several of them such as smart light bulbs, smart curtains, air conditioning systems, heaters or multimedia devices are present in multiple numbers in different rooms within the home. While some IoT devices are at a fixed position and rarely move (light bulbs, heaters, curtains, etc.) others are portable and nomadic by nature (portable speakers, vacuum cleaner robot, wearable devices, etc.).

Interactions with IoT devices can also be found in other types of environments and use cases as they are a main component of digital twins. For instance, AR can be used in smart building to assist security agents during their patrols and allow them to supervise the state of the building sensors as illustrated in Figure A.2.1-2. In industrial use cases, similar interactions for controlling and supervising manufacturing equipment can be developed.

Figure A.2.1-1: AR visual interfaces for controlling IoT devices in a living room (Source: Orange™)

Figure A.2.1-2: Accessing to air quality sensor data with AR in a security patrol scenario (Source: Orange™)

## A.2.2 	Potential use of spatial computing functions

In this use case, Spatial Description encompasses the positioning of IoT devices around the user. This information can be leveraged by Relocalization and World Tracking functions to accurately locate these devices in relation to the user’s position, allowing for the appropriate placement of AR content. Consequently, the system can identify which IoT devices are present in the same room as the user, enabling control through simple voice commands without the need to target specific devices. For instance, the user can say, “switch off the light,” to operate the main light in the room they are currently in.

Through Trackable Anchoring and Segmentation functions, the system can monitor the positions of IoT devices: adding new objects and correcting the positioning of existing ones. When combined with World Mesh Reconstruction, a complete digital twin of the real world can be created, allowing users to visualize their environment in VR or through non-immersive setups (desktop or mobile).

# A.3	Diminished reality

## A.3.1	Description

The following use case is inspired by the app “IKEA Kreativ”.

A user needs to visualize how items of furniture would look and how they would fit at home. They want to browse and try different products from an on-line catalog. The user doesn’t want to physically move anything around. To remove an item, they use a diminished reality application.

NOTE:  The intention of this use case is to diminish reality by overlaying inpainting on top of objects to hide.

The process is as follows:

- The user is invited to scan their environment with their AR device.

- The application guides the user through the scanning process to capture images of the user environment from different points of view.

- Captured images, along with associated metadata, are uploaded to an application server. The server performs segmentation of the scene and identifies all items present in the room so that they can be selected. For all segmented items, an inpainting process is applied and a patch to hide the item is generated.

- The user receives a capture of the room space; The captured space is segmented, and all the items present in the room are identified and can be selected. The user receives a view of their environment along with selectable objects (Figure A.3.1-1 (a)) and a set of patches to hide each of those objects.

- The user selects an item they want to remove. This item is hidden in real-time from their viewport (Figure A.3.1-1 (b)).

- The user places virtual furniture in place of the removed item (Figure A.3.1-1 (c)).

| (a) | (b) | (c) |
| --- | --- | --- |

Figure A.3.1-1: Clear and place furniture in IKEA Kreativ: (a) item selection; (b) hiding items; and (c) overlaying other furniture items (Source: https://www.ikea.com/gb/en/newsroom/stories/ikea-kreativ-launches-making-home-design-even-easier-and-more-accessible-for-the-many-pub71d09380/)

## A.3.2 	Potential use of spatial computing functions

In this use case, spatial computing functions are used to:

- construct a segmented view of the space, and

- hide and replace selected object by the background.

The segmentation of the space allows the selection of items. This can be done using a Segmentation and Labeling spatial computing function. A mask is applied to the selected object. The masked area is filled with background information (inpainting process), and the selected objects can then be hidden.

When rendering the scene, two methods can be applied to the removed objects:

- A set of images containing transparent areas and non-transparent areas is used. Non-transparent areas hide real objects.

- 3D objects are used to hide real objects and their shadows.

# A.4	XR Conference

## A.4.1	Description

The following use case is inspired from TR 26.928 [2] (clause 5.7 – XR conference).

Figure A.4.1-1, extracted from TR 22.856 (clause 5.3 – collaborative engineering), illustrates an example of an XR conference scenario.

Figure A.4.1-1: XR enabled collaborative and concurrent engineering in product design 
(Source: www.spatial.io)

This use case caters for an XR conference with multiple physically co-located and remote participants using XR to create a telepresence experience. The shared conference space is a virtual space that has the same layout as the physical space (digital twins) so that the physically present (local) and remote participants have a similar experience while moving in the space (e.g., constructed from sensor data).

Figure A.4.1-2 illustrates a system based on this use case. Virtual spaces and avatars are retrieved from the Application Server by the Conference Server. A Spatial Computing Server is used for constructing the 3D Model of the physical space. Remote participants would be seen as avatars within the XR experience located at their relative position in the shared space. During the conference, a local participant anchors a virtual screen on a wall to share a document to all participants.

The required video processing is located in the conference server (e.g., a media function) in Figure A.4.1-2. The spatial computing function may perform media and/or metadata processing that is required to place users consistently into a virtual environment.

The conversation/real-time XR stream shown in the figure is a mix of VR (remote user) or AR (local user) media, room layout, and mixed audio.

Figure A.4.1-2: XR Conference

## A.4.2 	Potential use of spatial computing functions

In this case, spatial computing functions are used to:

- Anchor the avatars and virtual assets in the AR scene for local participants at the beginning session.

- Anchor the virtual items (e.g., virtual screen) added during the session.

- Handle a 3D Model (layout of the physical space), including the creation of the 3D model in an offline process prior to the IMS session.

After anchoring, each user can be located and can interact with other participants (local and remote). As shown in Figure A.4.1-1, local and remote participants can interact with virtual assets in a shared experience.

Annex B:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-08 | SA4#129-e | S4-241607 | - | - | - | Initial draft skeleton of the TR. | 0.0.1 |
| 2024-08 | SA4#129-e | S4-241768 | - | - | - | SA4#129-e agreements: S4-241759. | 0.1.0 |
| 2024-11 | SA4#130 | S4-242017 | - | - | - | Post SA4#129-e Video SWG AHG agreements: S4aV240066 | 0.1.1 |
| 2024-11 | SA4#130 | S4-242108 | - | - | - | SA4#130 agreements: S4-241848, S4-241970, S4-241847, S4-242006 (clause 2), S4-242140, S4-242195, S4-242215, and S4-242106. | 0.2.0 |
| 2025-02 | SA4#131 | S4-250332 | - | - | - | SA4#131 agreements: S4-250321, S4-250371, and S4-250322. | 0.3.0 |
| 2025-04 | SA4#131-bis-e | S4-250524 | - | - | - | Post SA4#131 Video SWG AHG agreements: S4aV250033, S4aV250034. | 0.3.1 |
| 2025-04 | SA4#131-bis-e | S4-250721 | - | - | - | SA4#131-bis-e agreements: S4-250571, S4-250572, S4-250656, and S4-250657. | 0.4.0 |
| 2025-05 | SA4#132 | S4-251139 | - | - | - | SA4#132 agreements: S4-250984, S4-250986, S4-250964, S4-251070, S4-251071, S4-251157, S4-251158. | 0.5.0 |
| 2025-05 | SA#108 |  |  |  |  | Version 1.0.0 created by MCC to be sent to TSG SA for information | 1.0.0 |
| 2025-07 | SA4#133-e | S4-251545 | - | - | - | SA4#133-e agreements: S4-251416, S4-251488, S4-251526, and S4-251530. | 1.1.0 |