| 3GPP TR 26.926 V1.0.0 (2021-12) |  |
| --- | --- |
| Technical Report. |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems; (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	Overview and Scope	8

5	General Design and Modelling	8

5.1	Introduction	8

5.2	End-to-end Modelling of XR Traffic	8

5.2.1 	Architecture and System Model	8

5.2.2 	Interfaces and Traces	10

5.2.2.1	Introduction	10

5.2.2.2	Overview of Traces	10

5.2.3 	Benefits and Limitations	11

5.3 	P-Traces	11

5.4	5G System Model and Simulation	12

5.4.1 	5G System Model	12

5.4.2 	Core Network Model	14

5.4.3 	RAN Model	15

5.4.4 	Test Channels	16

5.5	Content Modelling	16

5.5.1 	Introduction	16

5.5.2 	V-Trace Generation	17

5.5.2.1	General	17

5.5.2.2	Impact of encoding parameters	17

5.5.2.3	V-Trace Generation	19

5.5.3 	V-Trace Format	19

5.5.4 	Other media types	20

5.6	Media Coding and Decoding Modelling	20

5.7	Content Delivery Modelling	20

5.8	Quality Metrics and Computation	20

5.9	Uplink Modeling	21

6	XR Split Rendering	21

6.1	Introduction	21

6.2	Reference System Design	21

6.3	Simulation System	21

6.4	Recommended Configurations	21

6.5	Traces and Statistical Models	21

6.6	Test Channel Results	21

7	Cloud Gaming	22

7.1	Introduction	22

7.2	Reference System Design	22

7.3	Simulation System	22

7.4	Recommended Configurations	22

7.5	Traces and Statistical Models	22

7.6	Test Channel Results	22

8	AR Conversational	22

9	Viewport Dependent VR Streaming	22

10	XR Distributed Computing	22

Annex <X> (informative): Change history	23

# Foreword

This Technical Specification|Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

Media services in general, but also in particular eXtended Reality (XR) and Cloud Gaming are some of the most important 5G media applications under consideration in the industry. XR is an umbrella term for different types of realities and refers to all real-and-virtual combined environments and human-machine interactions generated by computer technology and wearables. It includes representative forms such as Augmented Reality (AR), Mixed Reality (MR) and Virtual Reality (VR) and the areas interpolated among them.

On XR and Cloud Gaming traffic with high throughput, low latency and high reliability requirements, it is important to consider system aspects of such services. If the traffic requirements of the XR and Cloud Gaming service are flexible (e.g., the underlying architecture allows adaptation of content), then the capacity of the service can be studied by assessing the delay, throughput and reliability variations with increasing number of users in the system. In order to properly study this, detailed traffic characteristics are necessary.

Based on this, the present provides traffic models and quality evaluation methods for different media and eXtended Reality (XR) Services. In order to address this, generic modelling considerations are introduced and for different services reference designs, simulation models and suitable quality metrics are reported. This information permits to obtain accurate information on exact bitrate and delay requirements in uplink and downlink, develop detailed traffic traces, develop suitable statistical models for media and XR traffic and to evaluate the expected media quality of such services. The information may be used by other 3GPP groups in order to assess media quality for different configuration of 5G System parameters, as well as for evaluating the requirements in terms of QoS for XR and media services.

# 1	Scope

The present document provides traffic models and quality evaluation methods for different media and eXtended Reality (XR) Services. In order to address this, generic modelling considerations are introduced and for different services reference designs, simulation models and suitable quality metrics are reported. This information permits to obtain accurate information on exact bitrate and delay requirements in uplink and downlink, develop detailed traffic traces, develop suitable statistical models for media and XR traffic and to evaluate the expected media quality of such services. The information may be used by other 3GPP groups in order to assess media quality for different configuration of 5G System parameters, as well as for evaluating the requirements in terms of QoS for XR and media services.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 26.925, "Typical traffic characteristics of media services on 3GPP networks"

[3]	3GPP TR 26.928, "Extended Reality (XR) in 5G"

[5.1i]	3GPP TS 26.501, "System architecture for the 5G System (5GS)"

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

trace: a well-defined format to describe a sequence of timed data units together with relevant metadata for system simulation.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Overview and Scope

Editor’s Note: add the agreed information from permanent document

# 5	General Design and Modelling

## 5.1	Introduction

The system and RAN model used in this Technical Report for XR Traffic analysis follows the 5G System architecture as defined in TS 23.501 [5.1i]. In particular, the user plane aspects are considered as showing in Figure 5.1-1 for which an XR Server is in the external DN or in a trusted DN, and the XR Device is a 5G UE. The exchange of data is assumed to be carried out using the 5G System. Interfaces to the 5G Core functions for charging, QoS control, etc. are not considered in the below diagram, for details refer to TS 23.501.

Figure 5.1-1 Simplified Architecture for XR end-to-end services

In order to identify the traffic characteristics and the impacts of media and XR applications on 5G Systems and radio access networks, an appropriate modelling of both, the XR traffic as well as of the 5G System is desirable, also to identify the interaction of the two components. Note that the XR Server may be hosted in the cloud or may be hosted on an edge.

Details on the modelling of the 5G System and RAN as well as simulation parameters are provided in clause 5.2.

## 5.2	End-to-end Modelling of XR Traffic

### 5.2.1 	Architecture and System Model

In order to support modelling of end-to-end XR Traffic, an abstracted architecture and system model is introduced in Figure 5.2.1-1. This system is used as a baseline and may be refined for specific traffic classes. The system follows the basic building blocks from clause 5.1 but provides details for each of those aspects. In order to support simpler system modelling and simulation, the interfaces are supported by traces. Traces define a well-defined format to describe a sequence of timed data units together with relevant metadata for system simulation. An overview of traces is introduced in clause 5.2.2, details are provided in each of the functions.

In order to split the tasks across different 3GPP working groups and companies, an approach as introduced in Figure 5.2.1-1 is considered:

- A content model based on traces may be provided based on company input providing sufficient information on a rendered application, e.g. a game or a scene.

- Based on these content model traces, codec centric experts defines a content encoding and delivery model taking into account a system design, for example based on TR26.928 [3]. The system model includes encoding, delivery, decoding and also a quality definition, providing packet traces.

- 5G System and RAN simulations may be carried out by the 3GPP radio experts using the packet traces to simulate different traffic characteristics and evaluate different performance options.

An overview of the functions is provided below:

1)	Content Model: Provides typical characteristics for the XR content model for video, audio and potentially other data. This content is rendered for XR/CG consumption – for details refer to clause 5.5.

2)	Content Encoding Model: Provides the details for the content encoding in order to meet certain objectives. This includes the generation of sequences of application data units (slices, video frames, audio frames, etc.) and the incurred timestamp of each of the units is available. For details refer to clause 5.6.

3)	Content Delivery Model: provides details on content delivery, for example packetization, delay jitter, but possibly also more sophisticated models such as retransmission, TCP operations and so on. This also includes emulation of 5G Core Network. It produces packet traces (timestamp and size of each packet) based on traces of application data units. For details refer to clause 5.7.

4)	Core Network Model: This model emulates the core network behaviour in terms of delay, latency and packet losses in the core network, i.e. the interface between the UPF and gNB. For details refer to clause 5.4.

5)	Packet Radio Model: The packet radio model receives sequences/traces of packets at a given time and of a specified size. The packets may have additional metadata assigned that can potentially be used by the radio simulator. The packet traces are provided for multiple users and reflect typical traffic characteristics. The RAN simulator provides packet traces after delivery that reflect the occurred delays and losses for each user. For details refer to clause 5.4.

6)	Content Receiver Model: The content receiver converts the packet traces into application units taking into account delays and losses occurred. It may also model additional functions such as retransmissions or FEC, if applicable. For details refer to clause 5.7.

7)	Content Decoding Model: The decoding model uses the received application units to model the reconstruction of timed data (video frames, audio) considering delays, losses and also content model properties (error propagation, refresh data and so on). For details refer to clause 5.6.

8)	Quality Evaluation: A quality evaluation tool is provided that takes into account traces to compute different quality metrics based on packet, application unit and media quality. For details refer to clause 5.8.

9)	Uplink Model: A model for the uplink traffic in a similar fashion also providing packet traces. For details refer to clause 5.9.

Several of the functions are initialized by appropriate configurations. An overview of configurations is introduced in each of the functions, if needed.

Figure 5.2.1-1 General architecture and system model for XR Traffic

### 5.2.2 	Interfaces and Traces

#### 5.2.2.1	Introduction

Generally, traces include the following information:

-	Time series of data units

-	Size of each data units

-	Association to the application

-	Additional "metadata"

Traces can also directly be fed to a quality evaluation tool obtain the application quality based on the simulation as shown in clause 5.8. The benefit of traces compared to first order statistical model are as follows:

- traces to represent time correlations.

- traces can be used to assign additional metadata in the time series

- statistical models can be developed based on time series

- traces can be directly fed to quality evaluation tool

The trace formats are defined as CSV files and follow the recommendation in RFC4180 "Common Format and MIME Type for CSV Files". For CSV files, headers are added to identify the data types. From existing CSV types, the online tool https://csv.openbridge.dev/ has been used to

- Validate the csv file

- Create a schema for the files.

#### 5.2.2.2	Overview of Traces

Figure 5.2.1-1 shows different traces, associated to different interfaces. The following traces are defined:

- V-Traces: Video traces provide a time series of the statistics/complexity of a video frame. Such a V-Trace is expected to provide sufficient information such that a coding model can create suitable statistics statics for encoded video streams at the output. For details refer to clause 5.5.

- S-Traces:  Slice traces provide a time series of encoded video application data units, referred to as slices as known from H.264/AVC and H.265/HEVC suitable application data units as outputs from a video codecs. S-Traces are expected to provide sufficient information to the content delivery module to create IP packets. For details refer to clause 5.6.

- P-Traces: P Traces provide a time series of IP packets, possibly associated with different application flows and/or QoS Flows. In addition, they provide metadata information that may be beneficial for advanced delivery. For details refer to clause 5.3.

- P’-Traces have the same format as P-Traces, but in addition may include losses and delays observed due to the delivery over a network.

- S’-Traces are have the same format as S-Traces, but in addition may include information on losses and distortions due to the delivery over a network. For details refer to clause 5.7.

- V’-Traces: Video receive traces provide a time series of received video frames together with an associated encoding and delivery quality. For details refer to clause 5.6.

### 5.2.3 	Benefits and Limitations

Tbd

## 5.3 	P-Traces

Packet Traces (P-Traces) are defined to emulate the arrival of IP packets with associated metadata at certain network nodes. A P-Trace format together with semantics is defined in Table 5.3-1.

The first row just keeps an index of the packets.

The second to fourth data row information is considered to be available in a general delivery environment and are summarized as baseline parameters.

- availability_time: A relative time from the start of the trace when this packet is available in the delivery function

- size: the size of the packet in octets

- flow_id: an id to a flow in order to differentiate different QoS flows to identify a PDU session aligned with the QFI.

Table 5.3-1 P-Trace format

| Name | Type | Semantics |
| --- | --- | --- |
| number | BIGINT | Unique packet number in the delivery |
| Baseline parameters |  |  |
| availability_time | BIGINT | Availability time of packet for next processing step relative to start time 0 in microseconds (0 means lost). |
| size | BIGINT | packet size in bytes. |
| flow_id | BIGINT | an id to the flow in order to differentiate different QoS flows |
| Cross-Layer Parameter |  |  |
| buffer | BIGINT | The associated eye buffer 1=left 2=right In general, differentiates application traffic for different buffers, for example audio, video, left eye, right eye. |
| delay | BIGINT | Delay observed of the packet in the last processing step (-1 means lost) |
| render_timing | BIGINT | the rendering generation timing associated to the media included in the packet. |
| number_in_unit | BIGINT | The number of the packet within the unit (slice), start at 1 |
| last_in_unit | BIGINT | Indicates if this is the last packet in the slice/unit 0=no, 1=yes |
| type | BIGINT | The data type of the unit 0 unknown For video 1=intra 2=inter |
| importance | BIGINT | assigned relative importance information (higher number means higher importance) |

In addition, it is considered that an application may provide additional metadata per packet that if present, may be used by the delivery system. Different definitions are provided in the P-Trace format. The signaling of this data a 5G System is for further study. This information would not be available to RAN delivery in the incoming IP packets. These parameters are summarized as cross-layer parameters.

5GS through the QoS model may provide the ability that application streams are separated into different streams or the streams are at least marked accordingly. Details would need further discussion with SA2 on how application traffic and packet properties or stream properties can be matched to the 5G System architecture. This aspect may be considered, once it is identified that certain “cross-layer” markings make sense.

Based on this, it is suitable to use the basic packet trace structure as defined in Table 5.3-1 and that RAN1 can assume that RAN1 may have access to the above packet information. The P-Trace may be extended based on new findings.

For any system simulations, it is important that simulation are carried out only using the baseline parameters. Additional cross-layer parameters may be used as defined above. New packet-based markers may be added.

It is also considered that realization in the 5G System is only addressed once the benefits of an approach are identified.

## 5.4	5G System Model and Simulation

### 5.4.1 	5G System Model

In order to further model the 5G system, an alignment with the 5GS QoS model as defined in clause 5.7 of TS 23.501 is considered appropriate. The interface between the application domain and the 5G System is assumed to be based on QoS Flows. A QoS Flow is the finest granularity of QoS differentiation in the PDU Session. A QoS Flow ID (QFI) is used to identify a QoS Flow in the 5G System. User Plane traffic with the same QFI within a PDU Session receives the same traffic forwarding treatment (e.g. scheduling, admission threshold).

The QFI is carried in an encapsulation header on N3 without any changes to the e2e packet header and is used to uniquely identify a PDU Session. The principle for classification and marking of User Plane traffic and mapping of QoS Flows to AN resources is illustrated in Figure 5.7.1.5-1 of TS 23.501 and repeated in Figure 5.1-2.

Figure 5.2.1-1: The principle for classification and User Plane marking for QoS Flows and mapping to AN Resources (see TS 23.501 [5.1i], Figure 5.7.1.5-1)

In the downlink, incoming data packets are classified by the UPF based on the Packet Filter Sets of the DL PDRs in the order of their precedence. The UPF conveys the classification of the User Plane traffic belonging to a QoS Flow through an N3 User Plane marking using a QFI. The AN binds QoS Flows to AN resources (i.e. Data Radio Bearers of in the case of 3GPP RAN).

In UL, for relevant PDU Session of Type IP, the UE evaluates UL packets against the UL Packet Filters in the Packet Filter Set in the QoS rules based on the precedence value of QoS rules in increasing order until a matching QoS rule is found. The UE uses the QFI in the corresponding matching QoS rule to bind the UL packet to a QoS Flow. The UE then binds QoS Flows to AN resources.

TS 23.501 defines the following QoS characteristics:

- Clause 5.7.3.2: Resource type (Non-GBR, GBR, Delay-critical GBR)

-	A GBR QoS Flow uses either the GBR resource type or the Delay-critical GBR resource type. The definition of PDB and PER are different for GBR and Delay-critical GBR resource types, and the MDBV parameter applies only to the Delay-critical GBR resource type.

-	A Non-GBR QoS Flow uses only the Non-GBR resource type.

-	Clause 5.7.3.3: Priority Level;

-	The Priority Level associated with 5G QoS characteristics indicates a priority in scheduling resources among QoS Flows. The lowest Priority Level value corresponds to the highest priority.

-	Clause 5.7.3.4: Packet Delay Budget (including Core Network Packet Delay Budget);

-	The Packet Delay Budget (PDB) defines an upper bound for the time that a packet may be delayed between the UE and the N6 termination point at the UPF. For a certain 5QI the value of the PDB is the same in UL and DL. In the case of 3GPP access, the PDB is used to support the configuration of scheduling and link layer functions (e.g., the setting of scheduling priority weights and HARQ target operating points).

-	For GBR QoS Flows using the Delay-critical resource type, a packet delayed more than PDB is counted as lost if the data burst is not exceeding the MDBV within the period of PDB and the QoS Flow is not exceeding the GFBR. For GBR QoS Flows with GBR resource type not exceeding GFBR, 98 percent of the packets shall not experience a delay exceeding the 5QI's PDB.

-	Clause 5.7.3.5: Packet Error Rate;

-	The Packet Error Rate (PER) defines an upper bound for the rate of PDUs (e.g. IP packets) that have been processed by the sender of a link layer protocol (e.g. RLC in RAN of a 3GPP access) but that are not successfully delivered by the corresponding receiver to the upper layer (e.g. PDCP in RAN of a 3GPP access).

-	Thus, the PER defines an upper bound for a rate of non-congestion related packet losses. The purpose of the PER is to allow for appropriate link layer protocol configurations (e.g. RLC and HARQ in RAN of a 3GPP access). For every 5QI the value of the PER is the same in UL and DL. For GBR QoS Flows with Delay-critical GBR resource type, a packet which is delayed more than PDB is counted as lost, and included in the PER unless the data burst is exceeding the MDBV within the period of PDB or the QoS Flow is exceeding the GFBR.

-	Clause 5.7.3.6: Averaging window (for GBR and Delay-critical GBR resource type only);

- 	Each GBR QoS Flow shall be associated with an Averaging window. The Averaging window represents the duration over which the GFBR and MFBR shall be calculated (e.g. in the (R)AN, UPF, UE).

-	Clause 5.7.3.7: Maximum Data Burst Volume (for Delay-critical GBR resource type only).

-	Each GBR QoS Flow with Delay-critical resource type shall be associated with a Maximum Data Burst Volume (MDBV).

-	MDBV denotes the largest amount of data that the 5G-AN is required to serve within a period of 5G-AN PDB.

-	Every standardized 5QI (of Delay-critical GBR resource type) is associated with a default value for the MDBV (specified in QoS characteristics Table 5.7.4.1). The MDBV may also be signalled together with a standardized 5QI to the (R)AN, and if it is received, it shall be used instead of the default value.

-	The MDBV may also be signalled together with a pre-configured 5QI to the (R)AN, and if it is received, it shall be used instead of the pre-configured value

### 5.4.2 	Core Network Model

The core network is expected to operate on constant bitrate channel from the UPF to the gNB.

In the model showing in Figure 5.4.2-1, the core network model gets as input a P-Trace and delivers a P-Trace. Packets provided to the core network are expected to be delivered from the UPF to the gNB at a constant bitrate of bitrate R, whereby each packet is delivered at a delivery time being the later of the two

- The arrival time at the UPF

- The delivery end time of the previous packet

The arrival time at the gNB is modelled as the sum of the delivery time and the packet size divided by the bitrate R.

The core network delivery is considered to be error-free.

Figure 5.4.2-1 Core Network Model

The modelling of such a delay is for example show in Figure 5.4.2-2 for different packet size models.

Figure 5.4.2-2 Packet size and latency addition in core network model

The configuration parameters for the core network model are

-	the input packet trace

-	the core network bitrate

- 	the output packet trace

### 5.4.3 	RAN Model

The RAN delivery may be modelled by a function that receives P-Traces as input for each simulated user as shown in Figure 5.4.3-1 and produces a P’-Trace at the output for the primary user. The other users are only used from statistical competing traffic. Configuration parameters for the simulation are left to RAN experts.

Figure 5.4.3-1 Packet Radio Network Model

However, the usage of traces for RAN simulations is complex and there is a preference to operate with statistical models that emulate the arrival time and size of the packets. The main reason is for example, that RAN1 needs to simulate multiple cells at the same time, for example 63, for interference considerations. A statistical model is preferred.

### 5.4.4 	Test Channels

P-Traces as produced by XR applications may be used by RAN for complex RAN simulations. However, at the same time it is important to evaluate different XR application and encoding configurations that result in a P-Trace and then compare different settings. For this purpose, test channels are defined addressing two aspects:

-	Permit to emulate typical radio conditions in terms delays and losses.

-	Permit to evaluate the application quality for different representative radio conditions.

The test channel aligns with the 5G System and QoS Model as introduced in clause 5.4.1.

A test channel is defined by the following models:

-	Packet Error Rate:

-	Definition: the rate of PDUs (e.g. IP packets) that have been processed by the sender of a link layer protocol (e.g. RLC in RAN of a 3GPP access) but that are not successfully delivered by the corresponding receiver to the upper layer (e.g. PDCP in RAN of a 3GPP access).

-	Model: iid loss model independent of the packet size with parameter PLR

-	Packet Delay:

-	Definition: the time that a packet may be delayed between the UE and the N6 termination point at the UPF.

-	Model: iid distributed latency between 0 and a max_delay

Other parameters and modelling aspects for test channels are for further study.

## 5.5	Content Modelling

### 5.5.1 	Introduction

In order to evaluate the traffic characteristics and the quality of XR applications, it is relevant to operate with realistic source data. Preferably, the traffic of existing services is evaluated, for example taking P-Traces from existing XR services. However, in doing so, there are limitations

- in accessing the data

- in terms of legal restrictions on providing such data

- in terms of making use of such data for quality evaluation

- in terms of understanding the structure and details of the contained traffic

- in terms of understanding the options and impact of different system configurations

- and for other reasons

Based on this, an approach is proposed in this report as shown in Figure 5.5.1-1.

Figure 5.5.1-1 Content Modelling Approach

It is assumed that the raw media data is generated based on an XR/game session for one or several popular games, using typical pose traces and interactions, for example by a human playing the game. This information may be running over several minutes or even hours in order to create sufficiently representative statistics.

The output of the XR rendering engine is video representing texture, primitive and eye buffers.

For service and applications not relying on XR engines, other approaches may be taken. This is for further study and discussed as part of the specific traffic characteristics.

### 5.5.2 	V-Trace Generation

#### 5.5.2.1	General

The basic idea behind the generation of V-Traces is motivated by two main aspects:

1) 	Handling of several minutes or hours of raw data from a game engine is too significant. A statistical version is preferred in order to properly handle the amount of data

2)	Representative games are typically attached with copyright and licensing terms and hence, the raw media data cannot be shared publicly.

Based on this V-Trace generation has been initiated by

-	using a representative game

-	using a repeatable pose and interaction trace for the game

-	using a high-quality output of a game engine for each eye buffer, encoded for example in H.264(AVC)

-	decode these video sequences and store this raw data for proper model encoding to generate a trace. As an example, the decoded video sequence is 2K x 2K at 120 fps for each source buffer.

A shorter version of the raw data is then encoded in with different parameters to create some statistical output that allows to estimate the performance of a video codec when used with different configurations. For this purpose, initially a set of encoding parameters are used to understand the impact of each of the parameters for the resulting bitrate and quality. This information is then used for the modelling. Details of these results are provided in clause 5.5.2.2.

Finally, the longer sequence is encoded only with a small subset of parameters in order to generate the V-Trace. Details are provided in clause 5.5.2.3.

#### 5.5.2.2	Impact of encoding parameters

A shorter version of the sequence is sent through a model encoder for identifying the impacts of different parameter settings. The following x.265 parameters are used resulting in total with 24 different configurations. The configurations take into account different frame rates, different quality factors, an all intra and an all inter predicted sequence, as well as no slices or 8 slices, as follows:

./x265 --input input.yuv --input-res 2048x2048 --preset medium --fps [30,60] --crf [22, 28,34] --keyint [-1,1] --slice [1,8] --csv /csv_analfiles/[i,p]_[30,60]_[22,28,34]_[1,8].csv --csv-log-level 2 --log-level full  --rc-lookahead 0 --no-deblock  --bframes 0  --frames 480 --psnr --ssim  --output /output/xxxx.h265

A summary of the bitrates and resulting quality is provided in Table 5.5.2-1 for each of the 24 parameter settings.

Table 5.5.2-1 Bitrates and resulting quality for different configurations

| Mode | Frame rate | CRF | Slice | Data Rate in Mbit/s | Average QP | Average Rate Factor | Y PSNR | U PSNR | V PSNR | YUV PSNR | SSIM | SSIM(dB) | Frames |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I | 60 | 22 | 1 | 43.77 | 27.29 | 19.09 | 42.58 | 46.58 | 46.12 | 43.52 | 0.979 | 16.82 | 480 |
| I | 60 | 22 | 8 | 44.16 | 27.29 | 19.09 | 42.57 | 46.57 | 46.11 | 43.51 | 0.979 | 16.77 | 480 |
| P | 60 | 22 | 1 | 14.14 | 23.82 | 21.99 | 43.46 | 48.27 | 47.83 | 44.60 | 0.983 | 17.80 | 480 |
| P | 60 | 22 | 8 | 29.64 | 23.71 | 21.99 | 43.37 | 47.94 | 47.49 | 44.45 | 0.983 | 17.57 | 480 |
| I | 60 | 28 | 1 | 21.56 | 33.30 | 25.09 | 39.07 | 44.11 | 43.64 | 40.27 | 0.960 | 14.00 | 480 |
| I | 60 | 28 | 8 | 21.85 | 33.30 | 25.09 | 39.06 | 44.11 | 43.64 | 40.26 | 0.960 | 13.98 | 480 |
| P | 60 | 28 | 1 | 3.86 | 30.17 | 27.99 | 40.36 | 46.12 | 45.57 | 41.73 | 0.972 | 15.51 | 480 |
| P | 60 | 28 | 8 | 12.33 | 29.91 | 27.99 | 40.13 | 45.48 | 44.96 | 41.40 | 0.969 | 15.07 | 480 |
| I | 60 | 34 | 1 | 10.55 | 39.35 | 31.09 | 35.68 | 42.62 | 42.16 | 37.36 | 0.928 | 11.44 | 480 |
| I | 60 | 34 | 8 | 10.76 | 39.34 | 31.09 | 35.65 | 42.63 | 42.17 | 37.34 | 0.928 | 11.43 | 480 |
| P | 60 | 34 | 1 | 1.30 | 36.75 | 33.99 | 37.16 | 43.84 | 43.32 | 38.77 | 0.949 | 12.94 | 480 |
| P | 60 | 34 | 8 | 5.59 | 36.20 | 33.99 | 36.85 | 42.99 | 42.47 | 38.32 | 0.942 | 12.40 | 480 |
| I | 30 | 22 | 1 | 29.43 | 24.88 | 19.28 | 44.08 | 47.70 | 47.27 | 44.93 | 0.984 | 17.99 | 250 |
| I | 30 | 22 | 8 | 29.66 | 24.87 | 19.28 | 44.07 | 47.70 | 47.26 | 44.92 | 0.984 | 17.91 | 250 |
| P | 30 | 22 | 1 | 12.78 | 21.87 | 22.18 | 44.43 | 48.95 | 48.51 | 45.50 | 0.986 | 18.41 | 250 |
| P | 30 | 22 | 8 | 20.59 | 21.82 | 22.18 | 44.42 | 48.72 | 48.29 | 45.44 | 0.985 | 18.25 | 250 |
| I | 30 | 28 | 1 | 14.54 | 30.88 | 25.27 | 39.07 | 44.11 | 43.64 | 40.27 | 0.960 | 14.00 | 250 |
| I | 30 | 28 | 8 | 14.71 | 30.88 | 25.27 | 39.06 | 44.11 | 43.64 | 40.26 | 0.960 | 13.98 | 250 |
| P | 30 | 28 | 8 | 3.82 | 28.07 | 28.17 | 41.23 | 46.69 | 46.19 | 42.53 | 0.976 | 16.11 | 250 |
| P | 30 | 28 | 8 | 8.34 | 27.94 | 28.17 | 41.09 | 46.17 | 45.67 | 42.30 | 0.974 | 15.80 | 250 |
| I | 30 | 34 | 1 | 7.14 | 36.91 | 31.26 | 37.07 | 43.19 | 42.73 | 38.54 | 0.943 | 12.44 | 250 |
| I | 30 | 34 | 8 | 7.26 | 36.91 | 31.26 | 37.05 | 43.20 | 42.74 | 38.53 | 0.943 | 12.43 | 250 |
| P | 30 | 34 | 1 | 1.29 | 34.38 | 34.16 | 38.09 | 44.36 | 43.83 | 39.59 | 0.957 | 13.62 | 250 |
| P | 30 | 34 | 8 | 3.71 | 34.11 | 34.16 | 37.86 | 43.57 | 43.12 | 39.23 | 0.952 | 13.21 | 250 |

An analysis of the bitrate decrease per CRF/QP increase is shown in Table 5.5.2-2.

Table 5.5.2-2 Bitrate decrease per CRF/QP increase

| 1 Slices |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I only |  |  |  |  | P only |  |  |  |
| 22->28/60 | 28->34/60 | 22->28/30 | 28->34/30 |  | 22->28/60 | 28->34/60 | 22->28/30 | 28->34/30 |
| 2.03 | 2.04 | 2.02 | 2.04 |  | 3.85 | 3.23 | 3.46 | 3.18 |
| 8 Slices |  |  |  |  |  |  |  |  |
| I only |  |  |  |  | P only |  |  |  |
| 22->28/60 | 28->34/60 | 22->28/30 | 28->34/30 |  | 22->28/60 | 28->34/60 | 22->28/30 | 28->34/30 |
| 2.02 | 2.04 | 2.02 | 2.04 |  | 3.85 | 3.23 | 3.46 | 3.18 |

The bitrate increase for I-frames vs P-frames for different configurations is shown in Table 5.5.2-3.

Table 5.5.2-3 Bitrate decrease per CRF/QP increase

| Frame | I/P 22/60/1 | I/P 28/60/1 | I/P 34/60/1 | I/P 22/60/8 | I/P 28/60/8 | I/P 34/60/8 |
| --- | --- | --- | --- | --- | --- | --- |
| Average | 3.17 | 6.10 | 10.03 | 1.51 | 1.81 | 1.81 |

Finally, the bitrate increase for 60fps vs 30fps is shown in Table 5.5.2-4.

Table 5.5.2-4 Bitrate increase for 60 fps vs. 30fps

| Frame | 22-1 | 28-1 | 34-1 | 22-8 | 28-8 | 34-8 |
| --- | --- | --- | --- | --- | --- | --- |
| Average | 1.83 | 2.13 | 2.28 | 1.39 | 1.38 | 1.36 |

From these results some initial conclusions:

- Slices as available in x265 being constrained across pictures are very costly if used without prediction across slice boundaries

- Prediction over 2 P-frames results roughly in factor 2

- Rate decrease per CRF/QP for I is consistently 2 for QP increase 6

- Rate decrease per CRF/QP for P is consistently between 3 and 4 for QP increase 6

- Bitrate increase for I frame coding when doubling frame rate is typically in the range of 2.

- Bitrate increase for P frame coding when doubling frame rate is typically in the range of 1.4.

#### 5.5.2.3	V-Trace Generation

Based on the above model findings, it considered to only generate two V-Traces for the entire sequence to keep data handling manageable as shown in Figure 5.5.2.3-1. One V-Trace is generated for Intra coding only, the other one is generated for predictive coding as follows:

./x265 --input input.yuv --input-res 2048x2048 --preset medium --fps [30,60] --crf 28 --keyint [-1,1] --slice 1 --csv /csv_analfiles/[i,p]_[60]_[22,28]_[1,8].csv --csv-log-level 2 --log-level full  --rc-lookahead 0 --no-deblock  --bframes 0  --frames 480 --psnr --ssim  --output /output/xxxx.h265

The V-Traces are combined to document relevant statistics for each frame.

Figure 5.5.2.3-1 V-Trace Generation

### 5.5.3 	V-Trace Format

For each frame in the video sequence, the information as documented in Table 5.5.3-1 is provided.

Table 5.5.3-1 V-Trace Format

| Name | Type | Semantics |
| --- | --- | --- |
| time_stamp_in_micro_s | BIGINT | Associated rendering time of the frame |
| encode_order | BIGINT | The display order of the frames |
| i_qp | BIGINT | Quantization Parameter decided for the I frame. |
| i_bits | BIGINT | Number of bits consumed by the I frame. |
| i_y_psnr | BIGINT | Peak signal to noise ratio for Y planes in dB and multiplied by 1000 for I frame. |
| i_u_psnr | BIGINT | Peak signal to noise ratio for U planes in dB and multiplied by 1000 for I frame. |
| i_v_psnr | BIGINT | Peak signal to noise ratio for V planes in dB and multiplied by 1000 for I frame. |
| i_yuv_psnr | BIGINT | Peak signal to noise ratio for weighted Y, U and V planes in dB and multiplied by 1000 for I frame. |
| i_ssim | BIGINT | Quality metric that denotes the structural similarity between frames for I frame. |
| i_ssim_d_b | BIGINT | Quality metric that denotes the structural similarity between frames in dB for I frame. |
| i_total_frame_time_ms | BIGINT | Total time spent to encode the frame for I frame. |
| p_poc | BIGINT | The display order of the frames for P |
| p_qp | BIGINT | Quantization Parameter decided for the P frame. |
| p_bits | BIGINT | Number of bits consumed by the P frame. |
| p_y_psnr | BIGINT | Peak signal to noise ratio for Y planes in dB and multiplied by 1000 for P frame. |
| p_u_psnr | BIGINT | Peak signal to noise ratio for U planes in dB and multiplied by 1000 for P frame. |
| p_v_psnr | BIGINT | Peak signal to noise ratio for V planes in dB and multiplied by 1000 for P frame. |
| p_yuv_psnr | BIGINT | Peak signal to noise ratio for weighted Y, U and V planes in dB and multiplied by 1000 for P frame. |
| p_ssim | BIGINT | Quality metric that denotes the structural similarity between frames for P frame. |
| p_ssim_d_b | BIGINT | Quality metric that denotes the structural similarity between frames in dB for P frame. |
| p_total_frame_time_ms | BIGINT | Total time spent to encode the frame for P frame. |
| intra | DOUBLE PRECISION | Percentage of intra Coding Units in P frame |
| merge | DOUBLE PRECISION | Percentage of merge Coding Units in P frame |
| skip | DOUBLE PRECISION | Percentage of skip Coding Units in P frame |
| inter | DOUBLE PRECISION | Percentage of inter Coding Units in P frame |

### 5.5.4 	Other media types

Editor’s Note: add the agreed information from permanent document

## 5.6	Media Coding and Decoding Modelling

Editor’s Note: add the agreed information from permanent document

## 5.7	Content Delivery Modelling

Editor’s Note: add the agreed information from permanent document

## 5.8	Quality Metrics and Computation

Editor’s Note: add the agreed information from permanent document

In summary, for proper quality evaluation, SA4 needs P’-traces to identify application quality.

Agreement 2: It is agreement

-	that SA4 develops quality models based on P’-traces

Test Channel Results

## 5.9	Uplink Modeling

Editor’s Note: add the agreed information from permanent document

# 6	XR Split Rendering

## 6.1	Introduction

Editor’s Note: add the agreed information from permanent document

## 6.2	Reference System Design

Editor’s Note: add the agreed information from permanent document clause 6.2.1, 6.2.2 and 6.2.3

## 6.3	Simulation System

Editor’s Note: add the agreed information from permanent document

## 6.4	Recommended Configurations

Editor’s Note: add the agreed information from permanent document

## 6.5	Traces and Statistical Models

Editor’s Note: add the agreed information from permanent document

## 6.6	Test Channel Results

Editor’s Note: add the agreed information from permanent document

# 7	Cloud Gaming

## 7.1	Introduction

Editor’s Note: add the agreed information from permanent document

## 7.2	Reference System Design

Editor’s Note: add the agreed information from permanent document

## 7.3	Simulation System

Editor’s Note: add the agreed information from permanent document

## 7.4	Recommended Configurations

Editor’s Note: add the agreed information from permanent document

## 7.5	Traces and Statistical Models

Editor’s Note: add the agreed information from permanent document

## 7.6	Test Channel Results

Editor’s Note: add the agreed information from permanent document

# 8	AR Conversational

Editor’s Note: add the agreed information from permanent document

# 9	Viewport Dependent VR Streaming

Editor’s Note: add the agreed information from permanent document

# 10	XR Distributed Computing

Editor’s Note: add the agreed information from permanent document

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-04 | SA4#113e | S4-210663 |  |  |  | Initial Version | 0.1.0 |
| 2021-05 | SA4#114e | S4-210894 |  |  |  | Version agreed during SA4#114e (adds S4-210750) | 0.2.0 |
| 2021-08 | SA4#115e | S4-211208 |  |  |  | Version agreed during SA4#115e (adds S4-211028) | 0.3.0 |
| 2021-11 | SA4#116e | S4-211208 |  |  |  | Version agreed during SA4#116e (adds S4-211028) | 0.4.0 |
| 2021-12 | SA#94-e | SP-21339 |  |  |  | For Presentation to plenary | 1.0.0 |