| 3GPP TR 26.955 V0.6.0 (2021-03) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group TSG SA; 5G Video Codec Characteristics (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	10

Introduction	11

1	Scope	12

2	References	12

3	Definitions of terms, symbols and abbreviations	16

3.1	Terms	16

3.2	Symbols	16

3.3	Abbreviations	16

4	Overview of video codec capabilities in 3GPP services in Release 16	17

4.1	Introduction	17

4.2	TV Video Profiles	18

4.3	VR Video Profiles	19

4.4	5G Media Streaming	19

4.4.1	Introduction	19

4.4.2	5GMS Downlink Streaming default profile	20

4.4.2.1	H.264 (AVC)	20

4.4.2.2	H.265 (HEVC)	20

4.4.2.3	Encapsulation format	20

4.4.3	5GMS Uplink Streaming default profile	20

4.4.3.1	H.265 (HEVC)	20

4.4.3.2	Encapsulation format	20

4.4.4	5GMS Television (TV) profile	20

4.4.4.1	H.264 (AVC)	20

4.4.4.2	H.265 (HEVC)	20

4.4.4.3	Encapsulation format	21

4.4.5	5GMS Downlink 360 Virtual Reality (VR) profile	21

4.4.5.1	H.264 (AVC)	21

4.4.5.2	H.265 (HEVC)	21

4.4.5.3	Encapsulation format	21

4.5	Multimedia Telephony Services over IMS	21

4.5.1	Introduction	21

4.5.2	H.264 (AVC)	21

4.5.3	H.265 (HEVC)	22

4.5.4	Encapsulation format	22

4.6	Messaging Services	22

4.7	Screen Content Coding	22

5	Test and Characterization Framework for Video Codecs	22

5.1	Overview	22

5.2	Reference Sequences	24

5.3	Anchors	24

5.4	Reference Software Tools	25

5.4.1	General	25

5.4.2	H.264/AVC	25

5.4.3	H.265/HEVC	26

5.5	Metrics	26

5.5.1	General	26

5.5.2	SDR Metrics	26

5.5.3	HDR Metrics	28

5.5.4	Anchor Tuple Metrics Reporting	28

5.6	Tests	29

5.7	Characterization	30

6	Relevant Scenarios	32

6.1	Introduction	32

6.2	Scenario 1: Full HD Streaming	32

6.2.1	Motivation	32

6.2.2	Description of the Anticipated Application	33

6.2.3	Source Format Properties	33

6.2.4	Encoding and Decoding Constraints	34

6.2.5	Performance Metrics	34

6.2.6	Interoperability Considerations	34

6.2.7	Reference Sequences	35

6.2.8	Anchor Definition	35

6.2.8.1	Overview	35

6.2.8.2	H.264/AVC Anchors	35

6.2.8.2.1	Overview	35

6.2.8.2.2	JM-01	36

6.2.8.3	H.265/HEVC Anchors	36

6.2.8.3.1	Overview	36

6.2.8.3.2	HM-0x	36

6.2.9	Anchor Results	36

6.2.10	Additional Information and Performance Data	36

6.3	Scenario 2: 4K-TV	36

6.3.1	Motivation	36

6.3.2	Description of the Anticipated Application	37

6.3.3	Source Format Properties	38

6.3.4	Encoding and Decoding Constraints	38

6.3.5	Performance Metrics	38

6.3.6	Interoperability Considerations	38

6.3.7	Reference Sequences	39

6.3.8	Anchor Definition	39

6.3.8.1	Overview	39

6.3.8.2	H.264/AVC Anchors	39

6.3.8.3	H.265/HEVC Anchors	39

6.3.8.3.1	Overview	39

6.3.8.3.2	HM-0x	40

6.3.9	Anchor Results	40

6.3.10	Additional Information and Performance Data	40

6.4	Scenario 3: Screen Content Scenario	40

6.4.1	Motivation	40

6.4.2	Description of the Anticipated Application	41

6.4.3	Source Format Properties	41

6.4.4	Encoding and Decoding Constraints	41

6.4.5	Performance Metrics	42

6.4.6	Interoperability Considerations	42

6.4.7	Reference Sequences	42

6.4.8	Anchor Definition	43

6.4.8.1	Overview	43

6.4.8.2	H.264/AVC Anchors	43

6.4.8.2.1	Overview	43

6.4.8.2.2	JM-0x	43

6.4.8.3	H.265/HEVC Anchors	44

6.4.8.3.1	Overview	44

6.4.8.3.2	HM-01: Main 10 Profile with no fixed Intra	45

6.4.8.3.3	HM-02: Main 10 Profile with fixed Intra every second	46

6.4.8.3.4	SCC-01: Screen Content Profile with no fixed Intra	46

6.4.8.3.5	SCC-02: Screen Content Profile with no fixed Intra	46

6.4.9	Anchor Results	47

6.4.10	Additional Information and Performance Data	47

6.5	Scenario 4: Messaging and Social Sharing	47

6.5.1	Motivation	47

6.5.2	Description of the Anticipated Application	48

6.5.3	Source Format Properties	50

6.5.4	Encoding and Decoding Constraints	50

6.5.5	Performance Metrics	50

6.5.6	Interoperability Considerations	50

6.5.7	Reference Sequences	51

6.5.8	Anchor Definition	51

6.5.8.1	Overview	51

6.5.8.2	H.264/AVC Anchors	51

6.5.8.2.1	Overview	51

6.5.8.2.2	JM-0X	51

6.5.8.3	H.265/HEVC Anchors	52

6.5.8.3.1	Overview	52

6.5.8.3.2	HM-0X	52

6.5.9	Anchor Results	52

6.5.10	Additional Information and Performance Data	52

6.6	Scenario 5: Online Gaming	52

6.6.1	Motivation	52

6.6.2	Description of the Anticipated Application	53

6.6.3	Source Format Properties	53

6.6.3.1	Introduction	53

6.6.3.2	Category A: Low/medium dynamicity with low/medium complexity.	54

6.6.3.3	Category B: games with high dynamicity and low/medium complexity.	54

6.6.3.4	Category C: games with high dynamicity and high complexity.	54

6.6.3.5	Category D: photo-realistic games or games based on natural images/video.	54

6.6.3.6	Category E: XR game content	54

6.6.3.7	Summary	54

6.6.4	Encoding and Decoding Constraints	55

6.6.5	Performance Metrics	56

6.6.6	Interoperability Considerations	56

6.6.7	Reference Sequences	56

6.6.8	Anchor Definition	57

6.6.8.1	Overview	57

6.6.8.2	H.264/AVC Anchors	57

6.6.8.2.1	Overview	57

6.6.8.2.2	tbd	57

6.6.8.3	H.265/HEVC Anchors	57

6.6.8.3.1	Overview	57

6.6.8.3.2	HM-01: Main 10 Profile with no fixed Intra	58

6.6.8.3.3	HM-02: Main 10 Profile with fixed Intra every second	59

6.6.8.3.4	SCC-01: Screen Content Profile with no fixed Intra	59

6.6.8.3.5	SCC-02: Screen Content Profile with no fixed Intra	59

6.5.9	Anchor Results	59

6.5.10

Additional Information and Performance Data	60

7	Characterization for Existing Codecs	60

7.1	Introduction	60

7.2	H.265/HEVC Characterization against H.264/AVC	60

8	Initial Information on new Codecs	60

8.1	Introduction	60

8.2	Versatile Video Coding (VVC)	60

8.2.1	Overview	60

8.2.2	External subjective tests results	61

8.2.3	Test Configurations and Results for VTM	61

8.2.3.1	Introduction	61

8.2.3.2	Scenario 1: Full HD Streaming	62

8.2.3.3	Scenario 2: 4K-TV	62

8.2.3.4	Scenario 3: Screen Content Scenario	62

8.2.3.4.1	Overview	62

8.2.3.4.2	Configurations	62

8.2.3.4.2.1	VTM-01: Main 10 Profile with no fixed Intra	62

8.2.3.4.2.2	VTM-02: Main 10 Profile with fixed Intra every second	63

8.2.3.4.3	Test Results	63

8.2.3.5	Scenario 4: Messaging and Social Sharing	63

8.2.3.6	Scenario 5: Online Gaming	63

8.2.3.6.1	Overview	63

8.2.3.6.2	Configurations	64

8.2.3.6.2.1	VTM-03: Main 10 Profile with no fixed Intra	64

8.2.3.6.2.2	VTM-04: Main 10 Profile with fixed Intra every second	64

8.2.3.6.3	Test Results	65

8.2.4	Characterization	65

8.3	Essential Video Coding (EVC)	66

8.3.1	Overview	66

9	Gaps and Optimization Potential	66

9.1	Identified Gaps and Deficiencies with Existing Codecs	66

9.2	Potential Requirements for New Codecs	66

10	Conclusions and Proposed Next Steps	67

Annex A: Scenario Template	67

A.1	Introduction	67

A.2	Template	67

Annex B: Data Formats and Metrics	68

B.1	Introduction	68

B.2

Raw Video Sequences	68

B.2.1 Overview	68

B.2.2 JSON Schema	69

B.2.3 Example	69

B.3	Encoded Video Sequences	70

B.2.1 Overview	70

B.3.2 JSON Schema	71

B.3.3 Example	71

Annex C: Candidate Reference Sequences	72

C.1	Introduction	72

C.2	 Gaming Test Sequences	72

C.2.1 	Overview	72

C.2.2	AOV	73

C.2.2.1	Introduction	73

C.2.2.2	Sequence properties	74

C.2.2.3	Copyright and license information	74

C.2.3	Baolei-Man	74

C.2.3.1	Introduction	74

C.2.3.2	Sequence properties	75

C.2.3.3	Copyright and license information	75

C.2.4	Baolei-Woman	75

C.2.4.1	Introduction	75

C.2.4.2	Sequence properties	76

C.2.4.3	Copyright and license information	76

C.2.5	Baolei-Balloon	76

C.2.5.1	Introduction	76

C.2.5.2	Sequence properties	77

C.2.5.3	Copyright and license information	77

C.2.6	Baolei-Yard	77

C.2.6.1	Introduction	77

C.2.6.2	Sequence properties	78

C.2.6.3	Copyright and license information	78

C.2.7	Jianling-Temple	78

C.2.7.1	Introduction	78

C.2.7.2	Sequence properties	79

C.2.7.3	Copyright and license information	79

C.2.8	Jianling-Beach	79

C.2.8.1	Introduction	79

C.2.8.2	Sequence properties	80

C.2.8.3	Copyright and license information	80

C.2.9	Heroes of the Storm	80

C.2.9.1	Introduction	80

C.2.9.2	Sequence properties	81

C.2.9.3	Copyright and license information	81

C.2.10	Project CARS	81

C.2.10.1	Introduction	81

C.2.10.2	Sequence properties	82

C.2.10.3	Copyright and license information	82

C.2.11	World of WarCraft: WoW	82

C.2.11.1	Introduction	82

C.2.11.2	Sequence properties	83

C.2.11.3	Copyright and license information	83

C.2.12	Minecraft	83

C.2.12.1	Introduction	83

C.2.12.2	Sequence properties	84

C.2.12.3	Copyright and license information	84

C.2.13	CS:GO	84

C.2.13.1	Introduction	84

C.2.13.2	Sequence properties	85

C.2.13.3	Copyright and license information	85

C.2.14	StarCraft	85

C.2.14.1	Introduction	85

C.2.14.2	Sequence properties	86

C.2.14.3	Copyright and license information	86

C.3	4K-TV Sequences	86

C.3.1 	SDR Category	86

C.3.1.1	Candidate Test Sequences	86

C.3.1.1.1	Introduction	86

C.3.1.1.2	Ultra Video Group Sequences	87

C.3.1.1.3	JVET Sequences	87

C.3.1.1.4	Netflix Sequences	87

C.3.1.1.5	EBU Sequences	88

C.3.1.1.6	SVT Sequences	88

C.3.1.1.7	4ever Sequences	88

C.3.1.1.8	CableLabs Sequences	89

C.3.1.2	Selected Test Sequences	90

C.3.1.2.1	Introduction	90

C.3.1.2.2	x265 Encoding	90

C.3.1.2.3	SI-TI Metrics	92

C.3.1.2.4	SDR Sequence Selection	94

C.3.1.3	Selected Sequences	96

C.3.1.3.1	Brest-Sedof	96

C.3.1.3.2	Rain Fruits	97

C.3.1.3.3	Park Joy	97

C.3.1.3.4	Soccer	98

C.3.1.3.5	Tunnel Flag	98

C.3.1.3.6	Boat	99

C.3.1.3.7	Fountain	99

C.3.1.3.8	Riverbank	100

C.3.2 	HDR Category	101

C.3.2.1	HDR Candidate Test Sequences	101

C.3.2.1.1	Introduction	101

C.3.2.1.2	AOM Sequences	101

C.3.2.1.3	CableLabs Sequences	102

C.3.2.2	Selected Test Sequences	103

C.3.2.2.1	Introduction	103

C.3.2.2.2	x265 Encoding	103

C.3.2.2.3	SI-TI Metrics	106

C.3.2.2.4	Codewords distribution	107

C.3.2.2.5	HDR Sequence Selection	110

C.3.2.3	Selected Sequences	112

C.3.2.3.1	Life Untouched	112

C.3.2.3.2	Meridian	113

C.3.2.3.3	Sol Levante	114

C.3.2.3.4	Cosmos	114

C.3.2.3.5	Elevator	115

C.3.2.3.6	Sparks	116

C.3.2.3.7	Nocturne	117

C.3.3 	HFR Category	117

C.4	 HD Test Sequences	117

C.4.1 	Overview	117

[C.4.2	CTA WAVE Tears of Steel	118

C.4.2.1	Introduction	118

C.4.2.2	Sequence properties	118

C.4.2.3	Copyright and license information	119

C.4.3	CTA WAVE Croatia	119

C.4.3.1	Introduction	119

C.4.3.2	Sequence properties	119

C.4.3.3	Copyright and license information	120

C.5	 Messaging Sequences	120

C.5.1 	Overview	120

C.5.2	Powder	120

C.5.2.1	Introduction	120

C.5.2.2	Sequence properties	121

C.5.2.3	Copyright and license information	121

C.6	 Screen Content Test Sequences	122

C.6.1 	Overview	122

C.6.2	Moving Text 2	122

C.6.2.1	Introduction	122

C.6.2.2	Source sequence properties	122

C.6.2.3	Copyright information	123

C.6.3	Text Mix Transitions	123

C.6.3.1	Introduction	123

C.6.3.2	Source sequence properties	123

C.6.3.3	Copyright information	124

C.6.4	Graphics Mix Simple	124

C.6.4.1	Introduction	124

C.6.4.2	Source sequence properties	124

C.6.4.3	Copyright information	125

C.6.5	Graphics Mix Transition	125

C.6.5.1	Introduction	125

C.6.5.2	Source sequence properties	125

C.6.5.3	Copyright information	126

Annex D: Reference Configurations	126

D.1	Introduction	126

D.2	 H.264/AVC JM19.0 Reference Configurations	126

D.2.1 	JM-01: Low-latency	126

D.3	 H.265/HEVC HM16.22 Reference Configurations	126

D.3.1 	HM-01: Low-latency without Intra	126

D.3.2 	HM-02: Low-latency with regular Intra	126

D.4	 H.265/HEVC SCM-8.8 Reference Configurations	126

D.4.1 	SCC-01: Low-latency without Intra	126

D.4.2 	SCC-02: Low-latency with regular Intra	126

Annex E Data Management and Hosting	127

E.1	Introduction	127

E.2	Reference Sequences	127

E.2.1 	Hosting	127

E.2.2 	Uploading	127

E.2.3 	Downloading	127

E.3	Anchors	128

E.3.1 	Hosting	128

E.3.2 	Uploading	129

E.3.3 	Downloading	129

E.4	Tests	129

E.4.1 	Hosting	129

E.5	Verification	130

Annex F: Software Package	130

F.1	Introduction	130

F.2	Readme	130

F.2.1	Quickstart	130

F.2.2	Using docker	131

F.2.3	Usage	131

F.2.3.1	General	131

F.2.3.2	encode	131

F.2.3.3	 decode	131

F.2.3.4	 metrics	131

F.2.4	Reference encoders	131

F.2.5	Anchor definition	132

F.2.5.1	example anchor.json :	132

F.2.5.2	Variant options to encoder CLI args mapping	132

F.2.5.3	Output	133

F.2.6	Raw video sequence description	133

F.2.7	Current limitations	133

F.2.8	Dependencies [metrics]	134

Annex G: Attachments	134

G.1	Introduction	134

Annex <X> (informative): Change history	135

For definitive guidance on drafting 3GPP TSs and TRs, see 3GPP TS 21.801 supplemented by the 3GPP web page http://www.3gpp.org/specifications-groups/delegates-corner/writing-a-new-spec.

Ensure all blue guidance text is removed before submitting the TS/TR to the TSG for approval.

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In drafting the TS/TR, pay particular attention to the use of modal auxiliary verbs! TRs shall not contain any normative provisions.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

Predictions on mobile video consumption are ever increasing. Different studies point to dominance of video traffic in 5G networks reaching from 65% total traffic in the short-term all the way to 90% by the end of the decade. Video is expected to be integral for services such as enhanced mobile media (such as mobile streaming services), home broadband and TV (for example in the context of 5G fixed wireless access services), immersive and interactive media in the context of eXtended Realities (XR) and cloud gaming as well as new media services from new verticals. This indicates that the user experience and efficiency of 5G networks will be heavily impacted by the quality of video compression technologies that are used with 5G services. Efficient video compression and decompression technologies required dedicated hardware for power and resource efficient real-time execution but are at the same time complex and costly in terms of implementation on integrated platforms. Hence, typically state-of-the-art video compression technologies last for several years and are used as generic service enablers for different applications and services, including traditional streaming and conversational services, but also new media services. This document analyzes the currently defined 3GPP-defined video compression technologies for their suitability for existing and emerging services in the context of 5G and identifies gaps and optimization potentials that would warrant the introduction of new video compression technologies.

# 1	Scope

The present document relevant interoperability requirements, performance characteristics and implementation constraints of video codecs in 5G services, and to characterize existing 3GPP video codecs, in particular H.264/AVC and H.265/HEVC in order to have a benchmark for the addition of potential future video codecs. For this purpose, the document:

-	Collects a summary of the video coding capabilities in 3GPP services.

-	Collects a subset of relevant scenarios for video codecs in 5G-based services and applications, including video formats (resolution, frame rates, color space, etc.), encoding and decoding requirements, adaptive streaming requirements.

-	Collects relevant and exemplary test conditions and material for such scenarios, including test sequences.

-	Defines performance metrics for such scenarios with focus on objective performance metrics.

-	Collects relevant interoperability functionalities and enabling elements for video codecs in different 5G services such as MTSI and Telepresence (i.e. RTP based conversational communications), or 5G media streaming (e.g. based on DASH/CMAF) supporting the identified scenarios.

-	Collects relevant criteria and key performance indicators for the integration of video codecs in 5G processing platforms, taking into account factors such as encoding and decoding complexity in the context of the defined scenarios.

-	Characterizes the existing codecs H.264/AVC and H.265/HEVC in the context of the above scenarios and document the findings in a consistent manner.

-	Identifies gaps and deficiencies of existing codecs in such use cases and derive requirements for potential new codecs.

-	Collects initial information on how new codecs under development in ISO/IEC SC29 WG11 (MPEG)/JVET (in particular including VVC and EVC) may meet the above criteria.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[3]	3GPP TS 26.116: "Television (TV) over 3GPP services; Video profiles".

[4]	3GPP TS 26.118: "3GPP Virtual reality profiles for streaming applications".

[5]	3GPP TR 26.925: "Typical traffic characteristics of media services on 3GPP networks".

[6]	3GPP TR 26.928: "Extended Reality over 5G".

[7]	Recommendation ITU-T H.264 (06/2017): "Advanced video coding for generic audiovisual services" | ISO/IEC 14496-10:2014/Amd.3 2016: "Information technology - Coding of audio-visual objects - Part 10: Advanced Video Coding".

[8]	Recommendation ITU-T H.265 (11/2019): "High efficiency video coding" | ISO/IEC 23008-2:2020: "High Efficiency Coding and Media Delivery in Heterogeneous Environments - Part 2: High Efficiency Video Coding".

[9]	2020 Mobile Internet Phenomena Report, accessible here: https://www.sandvine.com/download-report-mobile-internet-phenomena-report-2020-sandvine, February 2020.

[10]	2019 Ericsson Mobility Report, accessible here: https://www.ericsson.com/4acd7e/assets/local/mobility-report/documents/2019/emr-november-2019.pdf, November 2019.

[11]	T. Fautier, "New Codecs for 5G", DASH-IF Workshop on "Media Streaming meets 5G", December 2019, accessible here: https://dashif.org/docs/workshop-2019/04-thierry%20fautier%20-%20Harmonic%20Codec%20Comparison%205G%20Media%20Workshop_Final%20v3.pdf

[12]	Bitmovin Video Developer Report, accessible here: https://go.bitmovin.com/video-developer-report-2019, September 2019.

[13]	3GPP TS 26.511: "5G Media Streaming (5GMS); Profiles, codecs and formats".

[14]	Recommendation ITU-R BT.709-6 (06/2015): "Parameter values for the HDTV standards for production and international programme exchange".

[15]	Recommendation ITU-R BT.2020-2 (10/2015): "Parameter values for ultra-high definition television systems for production and international programme exchange".

[16]	Recommendation ITU-R BT.2100-1 (06/2017): "Image parameter values for high dynamic range television for use in production and international programme exchange".

[17]	“You can watch Netflix on any screen you want, but you’re probably watching it on a TV”, https://www.vox.com/2018/3/7/17094610/netflix-70-percent-tv-viewing-statistics

[18]	YouTube Revenue and Usage Statistics (2020), https://www.businessofapps.com/data/youtube-statistics/

[19]	Ultra HD Forum service tracker B2C, https://ultrahdforum.org/uhd-service-tracker/“

[20]	HIS Market “4K-TV and UHD: the whole picture”, https://cdn.ihs.com/www/pdf/4ktv-uhd-ebook.pdf

[21]	A. Mercat, M. Viitanen, and J. Vanne, “UVG dataset: 50/120fps 4K sequences for video codec analysis and development,” Accepted to ACM Multimedia Syst. Conf., Istanbul, Turkey, June 2020.

[21]	R. Jullian, Y. Chen, F. Galpin, E. François, M. Kerdranvat (InterDigital), “Extra results to JVET-N605 “Comparative study of video coding solutions VVC, AV1 and EVC versus HEVC””, document JVET-O0898, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

[22]	R. Jullian, Y. Chen, F. Galpin, E. François, M. Kerdranvat (InterDigital), “Comparative study of video coding solutions VVC, AV1 and EVC versus HEVC”, document JVET-N0605, 14th Meeting: Geneva, CH, 19–27 March 2019.

[23]	S. Iwamura, S. Nemoto, A. Ichigaya, “[AHG13] Compression performance analysis for 4K and 8K HLG test sequences”, document JVET-N0828, 14th Meeting: Geneva, CH, 19–27 March 2019.

[24]	A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video,” document JVET-P2011, 16th JVET meeting: Geneva, CH, 1–11 Oct. 2019.

[25]	Nick Zarzycki, "The Complete Guide to Social Media Video Specs in 2018", July 23, 2018, https://blog.hootsuite.com/social-media-video-specs/

[26]		YouTube Help, "Recommended upload encoding settings", https://support.google.com/youtube/answer/1722171?hl=en

[27]	Facebook Help Center, "What are the video format guidelines for live streaming on Facebook?", https://www.facebook.com/help/1534561009906955

[28]	Deepak Kumar, "All about TikTok video size [ Full Guide]-2020", February 2020, https://tiktoktip.com/tiktok-size/

[29]	Snapchat business center, "Single Image or Video Specifications", https://businesshelp.snapchat.com/en-US/article/top-snap-specs

[30]	ISO/IEC 23000-19: "Information Technology Multimedia Application Format (MPEG-A) – Part 19: Common Media Application Format (CMAF) for segmented media".

[31]	GSMA White Paper, "Chinese operators make major RCS commitment: Whitepaper", April 9, 2020, https://www.gsma.com/futurenetworks/latest-news/china-operators-make-major-rcs-commitment-whitepaper/

[32]	3GPP TS 26.140, "Multimedia Messaging Service (MMS); Media formats and codecs".

[33]	GSMA RCC.71, "RCS Universal Profile Service Definition Document", version 2.4, October 2019.

[34]	GSMA NG.114, "IMS Profile for Voice, Video and Messaging over 5GS", February 2020.

[35]		3GPP TS 26.223, "Telepresence using the IP Multimedia Subsystem (IMS); Media handling and interaction"

[36]		"NVIDIA VIDEO CODEC SDK", https://developer.nvidia.com/nvidia-video-codec-sdk

[37]		Microsoft Online Documentation, "Real-time media calls and meetings with Microsoft Teams", https://docs.microsoft.com/en-us/microsoftteams/platform/bots/calls-and-meetings/real-time-media-concepts

[38]	Recommendation ITU-T H.266 (07/2020): "Versatile Video Coding" | ISO/IEC 23090-3:2020: "Information technology — Coded representation of immersive media — Part 3: Versatile video coding".

[39]	Recommendation ITU-T H.274 (07/2020): "Versatile supplemental enhancement information for coded video bitstreams" | ISO/IEC 23002-7:2020 "Information technology — MPEG video technologies — Part 7: Supplemental enhancement information messages for coded video bitstreams".

[40]	JCTVC-AA1006, “Verification test report for HEVC screen content coding extensions” V. Baroncini, H. Yu, R. Joshi, S. Liu, X. Xiu, J. Xu (editors)- 2017-09-08

[41]	Intel Xe LP presented at Intel Architecture Day 2020. Slide 101 https://newsroom.intel.com/wp-content/uploads/sites/11/2020/08/Intel-Architecture-Day-2020-Presentation-Slides.pdf

[42]	N. Barman, S. Zadtootaghaj, S. Schmidt, M. G. Martini and S. Möller, "GamingVideoSET: A Dataset for Gaming Video Streaming Applications," 2018 16th Annual Workshop on Network and Systems Support for Games (NetGames), Amsterdam, Netherlands, 2018, pp. 1-6. DOI: 10.1109/NetGames.2018.8463362

[43]	N. Barman, S. Schmidt, S. Zadtootaghaj, M. G. Martini, and S. Möller. 2018. An Evaluation of Video Quality Assessment Metrics for Passive Gaming Video Streaming. In Proceedings of the 23rd Packet Video Workshop (PV '18). ACM,  Amsterdam, Netherlands, 2018, pp. 7-12. DOI: https://doi.org/10.1145/3210424.3210434

[44]	Technical Paper ITU-T HSTP-VID-WPOM (07/2020): "Working practices using objective metrics for evaluation of video coding efficiency experiments".

[45]	3GPP TR 26.902, "Video codec performance".

[46]	3GPP TS 26.244, "Transparent end-to-end packet switched streaming service (PSS); 3GPP file format (3GP)"

[47]	ISO/IEC 23001-17,  "Information technology — MPEG systems technologies — Part 17: Carriage of Uncompressed Video in ISOBMFF"

[48]	Recommendation ITU-T P.910 (04/2008): "Subjective video quality assessment methods for multimedia applications".

[49]	Ultra HD Forum Guidelines, https://ultrahdforum.org/wp-content/uploads/UHD-Guidelines-V2.3-final.pdf

[50]	ISO/IEC 23094-1, "Information technology — General video coding — Part 1: Essential video coding"

[51]	ISO/IEC 23094-4, " Information technology — General video coding — Part 4: Conformance and Reference software for Essential Video Coding".

[52] 	ISO/IEC JTC 1/SC 29/WG 04 output document N0047, "Report on Essential Video Coding compression performance verification testing for SDR Content", Online meeting, Jan. 2021. http://www.mpegstandards.org/wp-content/uploads/mpeg_meetings/133_OnLine/w20000.zip.

[53] 		ISO/IEC JTC 1/SC 29/WG 04 output document N0030, "Report on Essential Video Coding compression performance verification testing for HDR/WCG content", Online meeting, Oct. 2020. https://www.mpegstandards.org/wp-content/uploads/mpeg_meetings/132_OnLine/w19832.zip.

[54]	Z. Wang, E. P. Simoncelli, A. C. Bovik, "Multiscale structural similarity for image quality assessment" in the Proceedings of the Thirty-Seventh Asilomar Conference on Signals, Systems and Computers, Vol. 2, pp. 1398–1402, 2004.

[55]	Z. Wang, A. C. Bovik, H. R. Sheikh, E. P. Simoncelli, "Image quality assessment: from error visibility to structural similarity", IEEE Transactions on Image Processing, Vol. 13, No. 4, pp. 600–612, April 2004

[56]	Netflix, “VMAF – Video Multi-Method Assessment Fusion,” version 1.3.5, repository, Aug. 2020. https://github.com/Netflix/vmaf/tags

[57]	F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, "JVET common test conditions and software reference configurations for SDR video", Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 document JVET-M1010, 13th JVET meeting, Marrakech, Morocco, January 9-18 2019.

[58]	A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy, "JVET common test conditions and evaluation procedures for HDR/WCG video," Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 document JVET-P2011, 16th JVET meeting, Geneva, Switzerland, Oct. 2019.

[59]	https://github.com/Netflix/vmaf/blob/master/resource/doc/vmafossexec.md

[60]	IETF RFC4180, Common Format and MIME Type for CSV Files

[61]	JVET-T2020: “VVC Verification Test Report for Ultra High Definition (UHD) Standard Dynamic Range (SDR) Video Content”, Mathias Wien, Vottorio Baroncini, Output document, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 20th Meeting, by teleconference, 7 – 16 Oct. 2020.

[62]	Fraunhofer HHI, “Fraunhofer Versatile Video Encoder (VVenC),” version 0.1.0 (initial release), https://github.com/fraunhoferhhi/vvenc/tags, online, accessed Oct. 2020.

[63]	JVET-T0103:” Information on and analysis of the VVC encoders in the SDR UHD verification test”, Christian Helmrich, Benjamin Bross et al, Input document, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 20th Meeting, by teleconference, 7 – 16 Oct. 2020.

[64]	Recommendation ITU-T P.910 (2008), Subjective video quality assessment methods for multimedia applications.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3GPP codec: a video codec defined in a 3GPP specification.

Anchor: combination of a reference sequence, a reference software of 3GPP codec and codec configuration to provide a baseline for codec testing and characterization.

Anchor Bitstream: a bitstream resulting from the encoding of a reference sequence with a reference software and a codec configuration assigned to an anchor.

Anchor Tuple: several anchors that differentiate only by one parameter in the codec configuration to change the bitrate/quality.

Characterization: comparison of a codec under test with a 3GPP codec based on selected metrics.

Metric: A single number to provide a measurable quality of an anchor.

Test: combination of tools to test a codec for comparison with anchor, i.e. same reference sequence and comparable codec configuration.

Test Bitstream: a bitstream resulting from the encoding of a reference sequence with a reference software and a codec configuration assigned to a test.

Test Tuple: several tests that differentiate only by one parameter in the codec configuration to change the bitrate/quality.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AAC	Advanced Audio Coding

ABR	Adaptive BitRate

AOV	Arena Of Valor

ATSC	Advanced Television Systems Committee

AVC	Advanced Video Coding

AVCHD	AVC High Definition

AVI	Audio Video Interleave

BMFF	Based Media File Format

CABAC	Context-Adaptive Binary Arithmetic Coding

CAE	Content Aware Encoding

CBP	Constrained Baseline Profile

CBR	Constant BitRate

CGI	Computed Generated Imaginary

CHP	Constrained High Profile

CMAF	Common Media Application Format

CRA	Clean Random Access

CTU	Coding Tree Unit

DASH	Dynamic Adaptive Streaming over HTTP

DVB	Digital Video Broadcasting

EVC	Essential Video Coding

FPS	Frames Per Second

GOP	Group-Of-Pictures

HDMI	High-Definition Multimedia Interface

HDR	High Dynamic Range

HDTV	High Definition TeleVision

HEVC	High-Efficiency Video Coding

HFR	High Frame Rate

HLG	Hybrid Log-Gamma

HLS	HTTP Live Streaming

HMD	Head-Mounted Display

HRD	Hypothetical Reference Decoder

HTML	HyperText Markup Language

IDR	Instantaneous Decoder Refresh

JVET	Joint Video Experts Team

MKV	MatrosKa Video

MMO	Massive Multiplayer Online

MMORPG	MMO Role-Playing Game

MMS	Multimedia Messaging Service

MOBA	Multiplayer Online Battle Arena

MPD	Media Presentation Description

MPEG	Moving Pictures Expert Group

MSE	Mean Square Error

MTSI	Multimedia Telephony Service over IMS

OBS	Open Broadcaster Software

PSNR	Peak Signal to Noise Ratio

RAP	Random Acces Point

RCS	Rich Communication Services

RDPCM	Residual Differential Pulse Code Modulation

RGB	Red Green Blue

RPG	Role-Playing Game

RTMP	Real-Time Messaging Protocol

RTP	Realtime Transport Protocol

RTS	RealTime Strategy

SDK	Software Development Kit

SDR	Standard Definition Range

SEI	Supplemental Enhancement Information

SI 	Spatial perceptual Information

SSIM	Structural Similarity Index Measure

SVOD	Subscription Video On Demand

TI 	Temporal perceptual Information

UDP	User Datagram Protocol

UHD	Ultra High Definition

UVG	Ultra Video Group

VBR	Variable BitRate

VVC	Versatile Video Coding

# 4	Overview of video codec capabilities in 3GPP services in Release 16

## 4.1	Introduction

This clause summarizes the video coding capabilities in relevant existing 5G services as of Releease-16. Video codecs introduced in this clause are referred to as 3GPP video codecs.

As of today, two codecs are prominently referenced and available, namely H.264 (AVC) [7] and H.265 (HEVC) [8].

Both codecs are defined as part of the TV Video Profiles in 3GPP TS 26.116 [3] and are also the foundation of the VR Video Profiles in 3GPP TS 26.118 [4]. The highest defined profile/level combinations are:

-	H.264 (AVC) Progressive High Profile Level 5.1 [7] with the following additional restrictions and requirements:

o	the maximum VCL Bit Rate is constrained to be 120Mbps with cpbBrVclFactor and cpbBrNalFactor being fixed to be 1250 and 1500, respectively.

o	the bitstream does not contain more than 10 slices per picture

-	H.265 (HEVC) Main-10 Profile Main Tier Level 5.1 [8] without any restrictions

Both codecs are also defined for other 3GPP-based services. More details on the codec capabilities and the necessary interoperability requirements for different services are collected in the remainder of this clause.

## 4.2	TV Video Profiles

The TV Video Profiles in TS 26.116 [3] address coded representations of TV distribution signals up to UHD-1 phase 2. Table 4.2-1 provides an overview of the TV relevant formats considered in the context of 3GPP TV Video Profiles.

In the context of TV Video Profiles, the following aspect are defined:

-	Bitstream: A media bitstream that conforms to a video encoding format and certain Operation Point.

-	Operation Point: A collection of discrete combinations of different content formats including spatial and temporal resolutions, colour mapping, transfer functions, etc. and the encoding format.

-	Receiver: A receiver that can decode and render any bitstream that is conforming to a certain Operation Point.

Table 4.2-1: TV over 3GPP services Video Profile Operation Points (TS 26.116 [3])

| Operation Point name | Resolution format | Picture aspect ratio | Scan | Max. frame rate | Chroma format | Chroma sub-sampling | Bit depth | Colour space format | Transfer Characteristics |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H.264/AVC 720p HD | 1280 × 720 | 16:9 | Progressive | 30 | Y'CbCr | 4:2:0 | 8 | BT.709 | BT.709 |
| H.265/HEVC 720p HD | 1280 × 720 | 16:9 | Progressive | 30 | Y'CbCr | 4:2:0 | 8 | BT.709 | BT.709 |
| H.264/AVC Full HD | 1920 × 1080 | 16:9 | Progressive | 60 | Y'CbCr | 4:2:0 | 8 | BT.709 | BT.709 |
| H.265/HEVC Full HD | 1920 × 1080 | 16:9 | Progressive | 60 | Y'CbCr | 4:2:0 | 8; 10 | BT.709; BT.2020 | BT.709; BT.2020 |
| H.265/HEVC UHD | 3840 × 2160 | 16:9 | Progressive | 60 | Y'CbCr | 4:2:0 | 10 | BT.2020 | BT.2020 |
| H.265/HEVC Full HD HDR | 1920 x 1080 | 16:9 | Progressive | 60 | Y'CbCr | 4:2:0 | 10 | BT.2020 | BT.2100 PQ |
| H.265/HEVC UHD HDR | 3840 x 2160 | 16:9 | Progressive | 60 | Y'CbCr | 4:2:0 | 10 | BT.2020 | BT.2100 PQ |
| H.265/HEVC Full HD HDR HLG | 1920 x 1080 | 16:9 | Progressive | 60 | Y'CbCr | 4:2:0 | 10 | BT.2020 | BT.2100 HLG |
| H.265/HEVC UHD HDR HLG | 3840 x 2160 | 16:9 | Progressive | 60 | Y'CbCr | 4:2:0 | 10 | BT.2020 | BT.2100 HLG |

Each of the Operation Point is associated to the support of a video codec and its Profile, Level and Tier (for HEVC) combined with additional bitstream constraints defined in TS 26.116 [3], clause 4. The Table 4.2-2 summarizes the video codec profile, tier and level associated to each operation point.

Table 4.2-2: Operation point video codec Profile/Tier/Level

| Operation Point name | Video Codec | Profile | Tier | Level |
| --- | --- | --- | --- | --- |
| H.264/AVC 720p HD | AVC | High | - | 3.1 |
| H.265/HEVC 720p HD | HEVC | Main | Main | 3.1 |
| H.264/AVC Full HD | AVC | High | - | 4.2 |
| H.265/HEVC Full HD | HEVC | Main-10 | Main | 4.1 |
| H.265/HEVC UHD | HEVC | Main-10 | Main | 5.1 |
| H.265/HEVC Full HD HDR | HEVC | Main-10 | Main | 4.1 |
| H.265/HEVC UHD HDR | HEVC | Main-10 | Main | 5.1 |
| H.265/HEVC Full HD HDR HLG | HEVC | Main-10 | Main | 4.1 |
| H.265/HEVC UHD HDR HLG | HEVC | Main-10 | Main | 5.1 |

For TV Video profiles, interoperability with ISO BMFF based systems and the DASH Streaming is of most relevance. Hence, for a codec to be used in the context of TV Video Profiles, the following is defined in terms of interoperability:

- The receiver requirements on elementary stream level

- The encapsulation of an elementary stream into an ISO Base Media File Format track

- The provisioning of the media as part of DASH Adaptation Set to support seamless switching

- All MPD-level signalling for the codec to support capability discovery

For details, refer to TS 26.116 [3], clause 4 and clause 5.

## 4.3	VR Video Profiles

The VR profiles for streaming applications defined in TS 26.118 [4] address the coded representation of 360 VR distribution signals. Table 4.3-1 provides an overview of the 360 VR relevant formats considered in the context of 3GPP VR profiles. The VR profiles follow the same logic as the TV Video profiles, they represent a list of interoperability points that are amended by the 3GPP services such as 5GMS.

Table 4.3-1: High-level Summary of Operation Points

| Operation Point name | Decoder | Bit depth | Typical Original Spatial Resolution | Frame Rate | Colour space format | Transfer Characteristics | Projection |  | RWP | Stereo |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Basic H.264/AVC | H.264/AVC HP@L5.1 | 8 | Up to 4k | Up to 60 Hz | BT.709 | BT.709 | ERP w/o padding |  | No | No |  |
| Main H.265/HEVC | H.265/HEVC MP10@L5.1 | 8, 10 | Up to 6k in mono and 3k in stereo | Up to 60 Hz | BT.709 BT.2020 | BT.709 | ERP w/o padding |  | Yes | Yes |  |
| Flexible H.265/HEVC | H.265/HEVC MP10@L5.1 | 8, 10 | Up to 8k in mono and 3k in stereo | Up to 120 Hz | BT.709 BT.2020 | BT.709, BT.2100 PQ | ERP w/o padding CMP |  | Yes | Yes |  |
| Main 8K H.265/HEVC | H.265/HEVC MP10@L6.1 | 10 | Up to 8k in mono and 6k in stereo | Up to 60 Hz for 8K and 120 Hz for 4k | BT.709 BT.2020 | BT.709, BT.2100 PQ, | ERP w/o padding |  | Yes, but restricted to coverage | Yes |  |

## 4.4	5G Media Streaming

### 4.4.1	Introduction

5G Media Streaming (5GMS) services are associated with a series of 5GMS profiles each of which containing a set of capability requirements associated to a service scenario. Default profiles (for downlink and uplink streaming) are also defined in case no other profile is claimed to be supported. The detailed requirements for 5G Media Streaming profiles and their associated codec requirements are defined in 3GPP TS 26.511 [13]. The following clauses focus on video codec requirements and recommendations in 5GMS specification as well as the encapsulation format.

NOTE:	In some profiles the HD-HDR capability is used as the ability for a UE to present video signals with all the following features: at least Full-HD resolution, bit depth of at least 10, at least 60 frames per second, Wide Colour Gamut and High Dynamic Range.

### 4.4.2	5GMS Downlink Streaming default profile

#### 4.4.2.1	H.264 (AVC)

TS 26.511 [13] requires the support of H.264 (AVC) Progressive High Profile Level 3.1 [7] decoding, with the maximum VCL Bit Rate constrained to be 14 Mbps.

If the 5GMS client supports the reception of video and HD-HDR capabilities, then TS 26.511 [13] requires the support of H.264 (AVC) Progressive High Profile Level 4.0 [7] decoding.

#### 4.4.2.2	H.265 (HEVC)

TS 26.511 [13] recommends the support of H.265 (HEVC) Main Profile, Main Tier, Level 3.1 [8] decoding with no interlace support.

If the 5GMS client supports the reception of video and HD-HDR capabilities, then TS 26.511 [13] requires the support of H.265 (HEVC) Main10 Profile, Main Tier, Level 4.1 [8] decoding with no interlace support.

#### 4.4.2.3	Encapsulation format

TS 26.511 [13] requires the support of Common Media Application Format (CMAF) encapsulation as defined in ISO/IEC 23000-19 [30].

### 4.4.3	5GMS Uplink Streaming default profile

#### 4.4.3.1	H.265 (HEVC)

TS 26.511 [13] requires the support of H.265 (HEVC) Main10 Profile, Main Tier, Level 4.1[8] encoding with no interlace support.

#### 4.4.3.2	Encapsulation format

TS 26.511 [13] requires the support of Common Media Application Format (CMAF) as defined in ISO/IEC 23000-19 [30].

#### 4.4.4	5GMS Television (TV) profile

#### 4.4.4.1	H.264 (AVC)

TS 26.511 [13] requires the support of H.264/AVC 720p HD as specified in TS 26.116 [3], clause 4.4.2.6.

TS 26.511 [13] recommends the support of H.264/AVC Full HD as specified in TS 26.116 [3], clause 4.4.3.6.

NOTE:	Associated codec profiles and levels are listed in Table 4.2-2 of the present document.

#### 4.4.4.2	H.265 (HEVC)

TS 26.511 [13] recommends the support of H.265/HEVC 720p HD, H.265/HEVC Full HD, H.265/HEVC, H.265/HEVC Full HD, H.265/HEVC UHD HDR, H.265/HEVC Full HD HDR HLG and H.265/HEVC UHD HDR HLG as specified in TS 26.116 [3].

If the 5GMS client supports the Television (TV) profile and HD-HDR capabilities, TS 26.511 [13] requires the support of H.265/HEVC Full HD as specified in TS 26.116 [3].

NOTE:	Associated codec profiles and levels are listed in Table 4.2-2 of the present document.

#### 4.4.4.3	Encapsulation format

TS 26.511 [13] requires the support of DASH as defined in 3GPP TS 26.116 [3], clause 5.

NOTE:	Full alignment between DASH support in TS 26.116 [3] and CMAF support in TS 26.511 is still to be investigated.

### 4.4.5	5GMS Downlink 360 Virtual Reality (VR) profile

#### 4.4.5.1	H.264 (AVC)

TS 26.511 [13] requires the support of the Basic H.264/AVC Operation Point as defined in 3GPP TS 26.118 [4], clause 5.1.4.

NOTE:	Associated codec profiles and levels are listed in Table 4.3-1 of the present document.

#### 4.4.5.2	H.265 (HEVC)

TS 26.511 [13] recommends the support of the Main H.265/HEVC Operation Point as defined in TS 26.118 [4], clause 5.1.5.

TS 26.511 [13] indicates that the Flexible H.265/HEVC Operation Point as defined in TS 26.118 [4], clause 5.1.6,  may be supported.

TS 26.511 [13] indicates that the Main 8K H.265/HEVC Operation Point as defined in TS 26.118 [4], clause 5.1.7,  may be supported.

NOTE:	Associated codec profiles and levels are listed in Table 4.3-1 of the present document.

#### 4.4.5.3	Encapsulation format

TS 26.511 [13] requires the support of DASH as defined in 3GPP TS 26.118 [4], clause 5.2.

NOTE:	Full alignment between DASH support in TS 26.116 [3] and CMAF support in TS 26.511 is still to be investigated.

## 4.5	Multimedia Telephony Services over IMS

### 4.5.1	Introduction

The Multimedia Telephony Services over IMS (MTSI) specification 3GPP TS 26.114 [2] covers the media aspects of 3GPP conversational services. For MTSI services the UE is associated with both encoder and decoder requirements. Therefore, the support for a given video codec with its profile and level is interpreted as the ability to:

-	encode any video input which characteristics fit in the level limits and produce a bitstream conformant with the codec profile.

-	decode any bitstream in the codec/profile/level configuration.

NOTE:	Additional constraints related to conversational services such as decoding process, location of the parameter sets are defined in TS 26.114 [2], clause 5.2.2.

### 4.5.2	H.264 (AVC)

TS 26.114 [2], in clause 5.2.2 requires the support of H.264 (AVC) [7] Constrained Baseline Profile (CBP) Level 1.2.

TS 26.114 [2], in clause 5.2.2 also recommends H.264 (AVC) [7] Constrained High Profile (CHP) Level 3.1.

### 4.5.3	H.265 (HEVC)

TS 26.114 [2] requires the support of H.265 (HEVC) [8] Main Profile, Main Tier, Level 3.1 except for constrained terminals for which it is only recommended.

### 4.5.4	Encapsulation format

MTSI is built upon the Real Time Protocol (RTP) for which the payload formats are specified in TS 26.114 [2], clause 7.4.3.

## 4.6	Messaging Services

3GPP TS 26.140 [32] specifies the media types, formats and codecs for the MMS within the 3GPP system. The document extends to codecs for speech, audio, video, still images, bitmap graphics, and other media in general, as well as scene description, multimedia integration and synchronization schemes.

Specifically, for video, the following capabilities are defined:

-	TS 26.140 [32] requires the support of H.264 (AVC) [7] Constrained Baseline Profile (CBP) Level 1.3.

-	TS 26.140 [32] recommends the support of H.264 (AVC) [7] High Profile Level 3.1 with frame_mbs_only_flag=1 by MMS clients supporting HDTV video content at a resolution of 1280x720 (720p) with progressive scan at 30 frames per second.

-	TS 26.140 [32] also recommends the support of H.265 (HEVC) [8] Main Profile, Main Tier, Level 3.1.

NOTE:	The specification has not been updated since 2014.

## 4.7	Screen Content Coding

3GPP TS 26.223 [35] specifies a client for the IMS-based telepresence service supporting conversational speech, video and text transported over RTP. Telepresence is defined as a conference with interactive audio-visual communications experience between remote locations, where the users enjoy a strong sense of realism and presence between all participants (i.e. as if they are in same location) by optimizing a variety of attributes such as audio and video quality, eye contact, body language, spatial audio, coordinated environments and natural image size.

As specified in 3GPP TS 26.223 [35] clause 5.2, telepresence UEs are required to support:

-	H.264 (AVC) [7] Constrained High Profile (CHP), Level 3.1,

-	H.264 (AVC) [7] Constrained Baseline Profile (CBP), Level 1.2 (for interworking with MTSI clients),

-	H.265 (HEVC) [8] Main Profile, Main Tier, Level 4.1.

As specified in 3GPP TS 26.223 [35], Telepresence UEs are also recommended to support dedicated HEVC extensions defined specifically for such types of content:

-	H.265 (HEVC) [8] Screen-Extended Main, Main Tier, Level 4.1,

-	H.265 (HEVC) [8] Screen-Extended Main 4:4:4, Main Tier, Level 4.1.

# 5	Test and Characterization Framework for Video Codecs

## 5.1	Overview

This clause defines the characterization framework for video codecs for relevant 3GPP scenarios. For this purpose, the following is applied:

-	A set of relevant scenarios is defined. The scenarios reflect a typical application for video codecs in 5G systems and networks. The scenarios are introduced in clause 6 and may be extended in future versions of this document.

-	For each scenario, one or several reference sequences are defined that serve as the baseline for anchor generation.

-	For each test scenario, one or several anchors are defined and generated. For details on anchors, please refer to clause 5.3. For anchor generation, reference software tools are used as introduced in clause 5.4.

-	For each of the anchors, metrics are provided. Metrics are documented in detail in clause 5.5.

-	Tests for new codecs can be developed and generated. They are equivalent to anchor generation, but possibly for other codecs. Tests are introduced in clause 5.6.

-	Codecs are to be characterized against anchors. Characterization is documented in terms of expected bitrate savings for a codec, and may include additional comparison parameters such as complexity increase, etc. The basic characterization framework is introduced in clause 5.7.

An overview of the anchor generation framework and the anchor metrics is provided in Figure 5.1-1. An integral part of this Technical Report is the following information:

-	Formats to store

reference and anchor sequences

-	Reference sequences for each of the defined scenarios

-	Reference software encoders

-	Anchor configuration files

-	Anchor bitstreams in a well-defined anchor bitstream format

-	A anchor metric computation based on a reference sequence, anchor bitstream and an anchor sequence.

-	Conforming decoders to generate an anchor sequence from an anchor bitstream

-	Anchor metrics in a well-defined storage format.

NOTE: as the anchor sequences can be generated by conforming decoders, anchor sequences are not included in this document.

Figure 5.1-1 Anchor Generation Framework and Anchor Metrics Generation

## 5.2	Reference Sequences

This document provides reference sequences that are used to generate anchors and are also made available in order to generate test bitstreams for other codecs. Reference sequences are selected to be representative for a scenario.

Reference sequences are collected described in Annex C of this document along with their properties and their licenses. A format for raw reference sequences based on a JSON schema is defined in clause B.2.

Annex E describes how to upload new proposed reference sequences and how to download reference sequences.

## 5.3	Anchors

Anchors provide a baseline that a tested method can be compared against. Anchors defined in this specification use a codec/profile/level that exists in an existing 3GPP specification as introduced in clause 4.

Anchor tuples are collected to address different quality and bitrates that can then be used for evaluation over a larger set of operation points.

The following principle apply to anchor definitions:

-	Each scenario typically has several well-defined anchors

-	An anchor is a combination of:

-	Explanation on anchor relevance

-	Reference sequence

-	Reference encoder

-	Encoder configuration

-	Encoding complexity estimation, if available

-	Variable encoder configuration to create multiple quality/bitrate variants (using for example QP variations or other bitrate/quality evaluation tools).

-	Anchor tuples creating multiple variants, each including

-	Anchor bitstream

-	Anchor Metrics

-	Additional recommended anchor information includes

-	MD5 check sum of the complete reconstructed yuv file (anchor sequence)

-	Output picture log from reference encoder

-	Output picture log from reference decoder

Anchors and anchor tuples are an integral part of this document.

Anchor tuples should be created over a wide range of parameters to provide sufficient data and overlap with expected test results to support the generation of characterization results (see clause 5.7).

The workflow for the generation of anchor tuples is shown in Figure 5.3-1.

Figure 5.3-1: Anchor Tuple Generation Framework and Anchor Tuple Metrics Generation

Anchors are provided according to the format as defined in Annex B.3.

## 5.4	Reference Software Tools

### 5.4.1	General

Anchors bitstreams are generated with a reference software. The reference software is documented in order to permit repeatability of the anchor generation.

Reference configurations for reference encoders are provided in Annex D.

### 5.4.2	H.264/AVC

For H.264/AVC generated anchor bitstreams, the following reference software has been used:

-	JM19.0 H.264/MPEG-4 AVC Reference Software: The reference software for H.264/MPEG-4 AVC is called JM (Joint Test Model). The JM software is maintained and can be downloaded from in the repository https://vcgit.hhi.fraunhofer.de/jct-vc/JM and development versions are available. For JM-based H.264/MPEG-4 AVC anchors, version 19.0 is used except specified otherwise. The reference software supports the following 3GPP profiles

-	H.264 (AVC) Constrained Baseline Profile

-	H.264 (AVC) Progressive High Profile

-	others are tbd

### 5.4.3	H.265/HEVC

For H.265/HEVC generated anchor bitstreams, the following reference software has been used:

-	HM16.22 H.265/MPEG-H HEVC Reference Software: The reference software for H.265/MPEG-H HEVC is called HM (HEVC Test Model). The HM software is maintained and can be downloaded from in the repository https://vcgit.hhi.fraunhofer.de/jct-vc/HM and development versions are available. For HM-based H.265/MPEG-4 HEVC anchors, version 16.22 is used except specified otherwise. The reference software supports the following 3GPP profiles

-	H.265 (HEVC) Main Profile

-	H.265 (HEVC) Main-10 Profile

-	SCM8.8: Extensions for screen content coding are provided with the SCM (Screen Content Coding Model) software available here https://vcgit.hhi.fraunhofer.de/jct-vc/HM/-/tags/HM-16.21+SCM-8.8. The reference software supports the following 3GPP profiles and levels

- 	H.265 (HEVC) [8] Screen-Extended Main Profile

## 5.5	Metrics

### 5.5.1	General

Each anchor bitstream gets assigned multiple performance metrics, in particular:

-	the bitrate of the sequence, i.e. the sum of the size of all compressed pictures divided by duration of the sequence.

-	If Standard Dynamic Range (SDR) is used, then the metrics in clause 5.5.2 apply.

-	If High Dynamic Range (HDR) is used, then the metrics in clause 5.5.3 apply.

An overview of the metrics is provided below. These metrics are implemented in software scripts defined in Annex F. This software is used to compute and report all the metrics.

Editor’s Note:  At this stage the metrics are defined as the output of the reference software that generates the anchors. In an updated version of the TR, an independent metric computation tool will be provided that allows to generate the identical metrics based on the references sequence and the decoded test sequence.

Subjective evaluation of anchors streams is not excluded per se, but this specification does not define any recommended metric or method until now.

### 5.5.2	SDR Metrics

For standard dynamic range (SDR) sequences, the following metrics are used:

-	Peak-Signal to Noise Ratio (PSNRy) of luminance component, as specified in [44]:

For an individual frame, the mean square error is calculated between the luma channel decY of the decoded output image and the luma channel origY of the original image according to

MSE_Y=1W*Hy=0H-1x=0W-1decYx,y-origYx,y2,

where decY(x,y) and origY(x,y) are the luma sample values at position x,y of the decoded and original image. W and H are the width and height of the luma component, respectively. The luma PSNR value for the frame is then calculated as:

PSNRy= 10*log10255≪(bitDepth-8)2MSE_Y,

where bitDepth is the bit-depth of the input video.  For reporting, bitDepth shall be equal to 10 for all sequences.  All 8-bit content shall be converted to 10-bit input in the encoder by shifting 2 bits to the left and using the 10-bit PSNR calculation to report testing results. The max PSNR for a frame is fixed to 999.99 dB.

Then the PSNR per sequence is computed as the sum of individual frames PSNR divided by the number of frames in the sequence.

Three PSNR numbers are ordinarily calculated in this manner; one for luma (PSNRy), and two for chroma (PSNRu and PSNRv).

-	Weighted PSNR over color components (wPSNRyuv):

wPSNRyuv = (6*wPSNRy + wPSNRu + wPSNRv)/ 8

-	Structural similarity metric (SSIM and MS-SSIM), as specified in [54] [55] and [56]:

The multi-scale SSIM method is illustrated in Figure x. Taking the reference x and distorted y image signals as the input, the system iteratively applies a low-pass filter and downsamples the filtered image by a factor of 2. We index the original image as Scale 1, and the highest scale as Scale M, which is obtained after M-1 iterations. At the j-th scale, the contrast comparison and the structure comparison are calculated and denoted as Cj (x, y) and Sj (x, y), respectively. The luminance comparison is computed only at Scale M and is denoted as lM (x, y). The overall SSIM evaluation is obtained by combining the measurement at different scales.

Figure x: Multi-scale structural similarity measurement system (L:low-pass filter, 2↓: downsampling by 2)

The MS-SSIM between the original image, I, and the reconstructed image component, I’, is calculated as:

MS_SSIMI,I'=1Nk=1N[LM(Ik, Ik')]αMj=1M[Cj(Ik, Ik')]βj[Sj(Ik, Ik')]γj

LIk, Ik'=2μIkμIk'+C1μIk2+μIk'2+C1

CIk, Ik'=2σIkσIk'+C2σIk2+σIk'2+C2

SIk, Ik'=σIkIk'+C3σIkσIk'+C3

where Ik,Ik' are pixels of original and reconstructed frames, N is the number of pixels per frame, αM, βj, γj, C1, C2, C3 are the constants:

β1 = γ1= 0.0448, β2 = γ2 = 0.2856, β3 = γ3 = 0.3001, β4 = γ4 = 0.2363, and α5 = β5 = γ5 = 0.1333.

C1= (K1*maxValue)2

C2= (K2*maxValue)2

C3=C2/2,

K1 = 0.01, K2 = 0.03, maxValue =(1<<bitDepth)-1, and M=5.

Average values μIk, μIk' at pixels Ik and  Ik'  are computed as weighted sum of 11×11 neighbors with Gaussian weights. σIk, σIk' represent the variance at pixels Ik and Ik'.  σIkIk' is covariance of these two pixels.

See section 5.7, on the use of [59] for computation of MS-SSIM.

-	Video Multimethod Assessment Fusion (VMAF), as specified in [57].

See section 5.7, on the use of [59] to compute VMAF.

### 5.5.3	HDR Metrics

For high dynamic range (HDR) sequences, the following metrics are used:

-	PSNRL100, as specified in [44] and [58],

-	Weighted PSNR over color components (wPSNR), as specified in [44] and [58],

-	DE100, as specified in [44] and [58].

Editor’s Note: For now, we use the output of the HM test model encoder – addition to scripts will be provided based on a fix of the HDRTools.

### 5.5.4	Anchor Tuple Metrics Reporting

For each anchor tuple, the Metrics are reported in a single csv file as defined in IETF RFC 4180 [60].

Additional metrics may be reported, for example encoding times, etc.

Table 5.5.4-1 provides the result format for SDR/HLG.

Table 5.5.4-1 Result Format for Full HD Scenario for SDR/HLG

| Name | Type | Semantics |
| --- | --- | --- |
| parameter | BIGINT | the associated variation parameter as defined for the anchor, for example the QP |
| bitrate | DOUBLEPRECISION | The size of the file divided by the duration of the reference sequence in bit/s as defined in clause 5.5.1 with 2 decimal digits accuracy. |
| y_psnr | DOUBLEPRECISION | Peak signal to noise ratio for Y planes in dB as defined in clause 5.5.2 with 2 decimal digits accuracy. |
| u_psnr | DOUBLEPRECISION | Peak signal to noise ratio for U planes in dB as defined in clause 5.5.2 with 2 decimal digits accuracy. |
| v_psnr | DOUBLEPRECISION | Peak signal to noise ratio for V planes in dB as defined in clause 5.5.2 with 2 decimal digits accuracy. |
| ms_ssim | DOUBLEPRECISION | structural similarity between frames as defined in clause 5.5.2 with 4 decimal digits accuracy. |
| vmaf | DOUBLEPRECISION | Video Multimethod Assessment Fusion (VMAF) as defined in clause 5.5.2 with 2 decimal digits accuracy. |
| bitrate_log | DOUBLEPRECISION | The bitrate as documented by the encoder log. If not known, it is set to 0. |
| encode_time | DOUBLEPRECISION | Total time spent to encode the sequence with reference encoder in seconds. If not known, it is set to 0. |
| decode_time | DOUBLEPRECISION | Total time spent to decode the sequence with reference decoder in seconds. If not known, it is set to 0. |

Table 5.5.4-2 provides the result format for HDR.

Table 5.5.4-2 Result Format for HDR

| Name | Type | Semantics |
| --- | --- | --- |
| Parameter | BIGINT | the associated variation parameter as defined for the anchor, for example the QP |
| Bitrate | BIGINT | The size of the file divided by the duration of the reference sequence in bit/s as defined in clause 5.5.1. |
| y_psnr | BIGINT | Peak signal to noise ratio for Y planes in dB as defined in clause 5.5.2 and multiplied by 1000. |
| u_psnr | BIGINT | Peak signal to noise ratio for U planes in dB as defined in clause 5.5.2 and multiplied by 1000. |
| v_psnr | BIGINT | Peak signal to noise ratio for V planes in dB as defined in clause 5.5.2 and multiplied by 1000. |
| Wpsnr | BIGINT | Weighted peak signal to noise ratio for Y, U and V planes in dB as defined in clause 5.5.3 and multiplied by 1000. |
| psnrl100 | BIGINT | PSNRL100 as defined in clause 5.5.3 and multiplied by 1000 |
| de100 | BIGINT | DE100 as defined in clause 5.5.3 and multiplied by 1000 |
| bitrate_log | BIGINT | The bitrate as documented by the encoder log. If not known, it is set to 0. |
| encode_time | BIGINT | Total time spent to encode the sequence with reference encoder in seconds. If not known, it is set to 0. |

## 5.6	Tests

Tests may be executed to compare codecs not yet in 3GPP specifications against anchors defined in this specification. Tests, equivalently to anchors, are collected in tuples to address different quality and bitrates that can then be used for evaluation over a larger set of operation points.

A test is developed against an anchor and is a combination of:

-	The corresponding anchor, which includes

-	Scenario

-	Reference Sequence

-	Test encoder

-	Test encoder configuration that provides an equivalent setting to the anchor configuration

-	Encoding complexity estimation, if available

-	Variable test encoder configuration to create multiple quality/bitrate variants (using for example QP variations or other bitrate/quality evaluation tools).

-	Test tuples creating multiple variants, each including

-	Test bitstream

-	Test Metrics

-	Additional recommended test information includes

-	MD5 check sum of the complete reconstructed yuv file (reconstructed test sequence)

-	Output picture log for reference encoder

-	Output picture log for reference decoder

-	Tests are an integral part of the Technical Report

The generation of test tuples is shown in Figure 5.6-1.

Figure 5.6-1: Test Tuple Generation Framework and Test Tuple Metrics Generation

## 5.7	Characterization

Characterization is the comparison of a codec under test with an anchor based on the framework introduced in this clause. Characterization in this report is based on Bjöntegard-Delta (BD)-rate information according to [44].

A full characterization of a codec for a scenario against a 3GPP codec shall provide at least the following metrics

-	The BD-rate gain for each defined anchor tuple and each required metric

-	The average BD-rate gain across all anchors of the scenario for each required metric.

Figure 5.7-1: Characterization Framework

BD-Rate is computed according to the CTC method used in JVET and specified in [44] from the tools publicly available: Reference codec software, Excel file available in [57] for SDR and in [58] for HDR.

The Excel files include the VBS script bdrate( ) to compute the BD-Rate performance between a test codec and a reference from four rate-distortion points.

These excel files have been extended in the Random-Access and low delay tabs to contain new columns for the new metrics: VMAF, SSIM and MS-SSIM, in the SDR case only. The “SA4 extended excel files” for SDR and HDR are attached as S4-template-HDR.xlsx and S4-template-SDR.xlsx.

Editor’s Note: At this stage the BD-Rate is defined as result of the above excel sheets. In an updated version of the TR, an independent metric computation tool may be provided that allows to generate the BD-Rate values based on the metrics results.

For the computation of VMAF, SSIM and MS-SSIM, the C++ executable “vmafossexec” [59], open source provided by Netflix could be used (Licence BSD + Patent) (Note: a tag need to be defined for libvmaf and vmafossexec). MS-SSIM is computed in Vmafossexec with the default 11 Gaussian Window and default K1=0.01 and K2=0.03.

Here is the command line:

vmafossexec $VMAF_FMT $WIDTH $HEIGHT ref.yuv test.yuv $VMAFMODEL --thread 1 --psnr --ssim --ms-ssim --log metrics.vmaf

$VMAF_FMT: describe yuv subsampling (yuv420p10le or yuv420p8In10leOut)

$VMAFMODEL: vmaf_4k_v0.6.1.pkl (4K and more) or vmaf_v0.6.1.pkl (HD and lower res)

thread: 0 to use all threads available

Note: the VMAF executable allows to extract the psnr which could also be used to check if it matches reference software output.

Another optional method is provided, as described in Annex F, to compute the metrics automatically.  BD-Rate computation is supported by a script that uses anchor tuple and test tuple metrics to provide the characterization results as shown in Figure 5.7-1.

For details on BD-Rate computation, refer to [44].

# 6	Relevant Scenarios

## 6.1	Introduction

This clause collects relevant scenarios based on the template defined in Annex A. It also defines the anchors for each scenario based on the existing 3GPP codecs and profiles defined in clause 4. For each scenario, the following information is provided:

-	Motivation: provides a context for the scenario, why it is relevant for 5G video services.

-	Description of the Anticipated Application: This provides an overview on how the service may be done technically in a 5G system. It summarizes where encoders and decoders reside, what codecs are in use today and how the end-to-end system is working.

-	Source format properties: Provides an overview of relevant video source formats in terms of spatial an temporal resolutions, colour spaces, etc.

-	Encoding and Decoding Constraints: provides details on encoder settings and configurations for each of the 3GPP codecs mapped to the scenario.

-	Performance Metrics: Documents the relevant performance metrics for this scenario

-	Interoperability Considerations: Documents relevant interoperability requirements for a codec when used in the scenario, based on the known applications and protocols.

-	Reference Sequences: defines the reference sequences that are selected for this scenario in order to create anchors and anchor metrics. A justification is provided, why this sequence is selected.

-	Anchor Definition: provides details on how to generate the anchors for the scenario for each 3GPP codec using the selected reference sequences, the reference software and the codec configuration.

-	Anchor Results: provides the results for each anchor.

-	Additional information and performance data: provides additional information and performance data available externally.

## 6.2	Scenario 1: Full HD Streaming

### 6.2.1	Motivation

The 2020 Mobile Internet Phenomena Report from Sandvine [9] shows that mobile video downstream traffic accounts for more than 65% of the global application category traffic share.

According to Ericsson mobility report [10], video traffic in mobile networks is forecast to grow by around 30 percent annually through 2025 to account for three-quarters of mobile data traffic, from slightly more than 60 percent in 2019. The video traffic growth is driven by the increase of embedded video in many online applications, growth of video-on-demand (VoD) streaming services in terms of both subscribers and viewing time per subscriber, and the evolution toward higher screen resolutions on smart devices. All of these factors have been influenced by the increasing penetration of video-capable smart devices.

Furthermore, while UHD and 4K are trendy formats, the main application for mobile streaming is Full HD with 1080p at 50 or 60 frames per second is expected to be the format of choice for mobile streaming at scale. The distribution version may be down-sampled to support adaptive bitrate streaming, possibly with High Dynamic Range (HDR) support. For detailed discussion please refer to the presentation at the DASH-IF Workshop Dec 2019 [11].

In terms of distribution, while in the past, streaming video was delivered primarily via RTMP or RTP, fewer and fewer devices support these aging protocols each year. Instead, the latest web standards support built-in video playback and HTML5 is now by far the preferred method for video playback. And adaptive bitrate protocols dominate the distribution. According to the developer report [12], adaptive bitrate streaming through HLS/DASH, using the CMAF/DASH based segment formats, provide vast majority for streaming video. The distribution is used for On-Demand and Live Streaming.

### 6.2.2	Description of the Anticipated Application

In the context of 3GPP services, 5G Media Streaming [13] as well as the TV Video Profiles [3] are specifications addressing this streaming scenario. Both, 5G Media Streaming [13] and TV Video Profiles [3] builds on CMAF-based Segment formats and DASH distribution. From TS 26.116, the following operation points may be considered in scope of the Full HD Streaming Scenario (pending availability of appropriate test content):

-	H.265/HEVC Full HD HDR, see TS26.116 [3] clause 4.5.3.

-	H.264/AVC Full HD, see TS26.116 [3] clause 4.4.3.

-	H.265/HEVC Full HD, see TS26.116 [3] clause 4.5.5.

-	H.265/HEVC Full HD HDR HLG, see TS26.116 [3] clause 4.5.7.

These operation points are further informed by relevant operational experience with commercially available encoders and decoders.

The considered scenario is the distribution of content through DASH/CMAF based streaming. Important aspects that are expected to be considered when evaluating a codec in the context of this:

-	Quality and Coding Efficiency:

-	High and uninterrupted visual quality, taking into account the service constraints.

-	Any savings can provide significant benefits due to the expected large volume of the traffic either in quality or network utilization.

- 	Adaptive Bitrate streaming:

-	Multiple bit rates are provided, typically with a ladder of 30–50% to permit bandwidth adaptation. The use of constant bit rate (CBR) encoding maximises reuse of a common ladder of encoded representations across multiple distribution networks. The use of capped variable bit rate (VBR) encoding allows the bit rate to be varied according to the difficulty of the source material while maintaining the ability to distribute the encoded representations through distribution networks with fixed capacity. This also maximises reuse of a common ladder across multiple distribution networks.

-	CMAF Fragments of size typically in the range of 1–6s to permit seamless switching for bit rate adaptation.

-	Regular Random Access, typically every 1–2 seconds according to TS 26.116 [3]. To achieve clean switching in both sound and picture when moving between different encoded representations in the ladder, 3.84 seconds enables video segment boundaries to be aligned with an integer number of audio Access Units, if a 50fps video signal and 48kHz audio signal is used.

-	Encoding in this scenario is typically done as

-	Live and On-Demand distribution and encoding

- 	Server and Cloud-based Encoding

-	No specific encoding latency constraints

### 6.2.3	Source Format Properties

Table 6.2.3-1 provides an overview of the different source signal properties following the information from TS26.116 [3]. This information is used to select proper test sequences.

Table 6.2.3-1 Source Format Properties for different operation point

| Source Format Properties | H.264/AVC Full HD | H.265/HEVC Full HD | H.265/HEVC Full HD HDR | H.265/HEVC Full HD HLG |
| --- | --- | --- | --- | --- |
| Spatial resolutions |  | 1920 × 1080 (Permitted Distribution formats: 1920 × 1080, 1600 × 900, 1280 × 720, 960 × 540, 854 × 480, 640 × 360,426 × 240) |  |  |  |  |  |
| Chroma Format |  | Y'CbCr |  |  |  |  |  |
| Chroma Subsampling |  | 4:2:0 |  |  |  |  |  |
| Picture Aspect ratios |  | 16:9 |  |  |  |  |  |
| Frame rates |  | 24; 25; 30; 50; 60; 24/1.001; 30/1.001; 60/1.001 Hz |  |  |  |  |  |
| Bit Depth | 8 | 8, 10 | 10 | 10 |
| Colour space formats | BT.709 [14] | BT.709 [14]; BT.2020 [15] | BT.2020 [15] | BT.2020 [15] |
| Transfer Characteristics | BT.709 [14] | BT.709 [14]; BT.2020 [15] | BT.2100 [16] PQ | BT.2100 [16] HLG |

### 6.2.4	Encoding and Decoding Constraints

Table 6.2.4-1 provides an overview of encoding and decoding constraints for H.264/AVC Full HD and H.265/HEVC Full HD Profiles. This will support the definition of detailed test conditions.

Table 6.2.4-1 Encoding and Decoding Configurations

| Encoding and Decoding Constraints | H.264/AVC Full HD | H.265/HEVC Full HD |
| --- | --- | --- |
| Relevant Codec and Codec Profile/Levels according to TS26.116 and TS26.511. | H.264/AVC Progressive High Profile Level 4.2 [7] | HEVC/H.265 Main-10 Profile Main Tier Level 4.1 [8] |
| Random access frequency | 1 second, 3.84 seconds | 1 second, 3.84 seconds |
| Error resiliency requirements | None | None |
| Bit rates and quality configuration | QP, for example [20, 23, 26, 29] | QP for example [23, 25, 28, 31, 34] |
| Bit rate parameters (CBR, VBR, CAE, HRD parameters) | Fixed QP CBR 8–12 Mbit/s VBR capped at 12 Mbit/s others | Fixed QP CBR 5–8 Mbit/s VBR capped at 12 Mbit/s others |
| ABR encoding requirements (switching frequency, etc.) | 1 second ABR through multiple QPs | 1 second ABR through multiple QPs |
| Latency requirements and specific encoding settings | No latency requirements beyond RAP so picture reordering allowed | No latency requirements beyond RAP so picture reordering allowed |
| Encoding complexity context | real-time encoding, cloud-based encoding, offline encoding, etc. | real-time encoding, cloud-based encoding, offline encoding, etc. |
| Required decoding capabilities | H.264/AVC Progressive High Profile Level 4.2 [7] | HEVC/H.265 Main-10 Profile Main Tier Level 4.1 [8] |

### 6.2.5	Performance Metrics

All metrics as defined in clause 5.5 apply.

### 6.2.6	Interoperability Considerations

In order to use a codec in the context of 5G Media Streaming services in TS 26.511 and for TV Video profiles in TS 26.116, the following list provides a set of potentially relevant interoperability aspects for Full HD Streaming:

1.	The receiver requirements on elementary stream level, in particular the profile/level and additional considerations.

2.	The encapsulation of an elementary stream into an ISO Base Media File Format track

3.	The definition of a CMAF media profile.

4.	The static mapping of parameters to a DASH MPD, in particular to the MPD parameters, such as @mimeType, @codecs, etc

5.	The dynamic mapping of parameters to a DASH MPD from a CMAF Principal Header, in particular to the MPD parameters, such as @width, @height, etc.

6.	All MPD-level signalling for the codec to support capability discovery

7.	Encryption requirements and recommendations.

8.	Capability discovery options, for example mapping to HTML-5, MSE and media capability APIs.

9.	Source Buffer Initialization Requirements.

10.	Playback Requirements, for example by referencing CTA WAVE Specifications

11.	Relation to other specifications, such as in DVB, ATSC, MPEG, ETSI, etc.

For additional details, please refer to TS 26.116 and TS 26.511.

### 6.2.7	Reference Sequences

Table 6.2.7-1 provides the selected reference sequences for this scenario. Keys are defined to refer to the sequences in the context of the scenario. The sequences are named and a reference to the details of the sequence is provided. A justification is provided, why this sequence is selected.

Table 6.2.7-1 Reference Sequences for Full HD Scenario

| Key | Name | Reference | Justification/Comment |
| --- | --- | --- | --- |
| S1-R1 | WAVE-ToS01 | Annex C.3.2.2 | Good long test sequence and full resolution of 1080p. |
| S1-R2 | WAVE-ToS02 | Annex C.3.2.2 | Good long test sequence at subsampled resolution of 900p. |
| S1-R3 | WAVE-ToS03 | Annex C.3.2.2 | Good long test sequence at subsampled resolution of 720p. |
| S1-R4 | WAVE-CRO01 | Annex C.3.3.2 | Good HDR Test sequence at full resolution of 1080p. |
| S1-R5 | WAVE-CRO02 | Annex C.3.3.2 | Good HDR Test sequence at subsampled resolution of 900p. |
| S1-R6 | WAVE-CRO03 | Annex C.3.3.2 | Good HDR Test sequence at subsampled resolution of 720p. |

Editor’s Note: More sequences need to be added.

### 6.2.8	Anchor Definition

#### 6.2.8.1	Overview

This clause provides details on how to generate the anchors for the Full HD Scenario.

#### 6.2.8.2	H.264/AVC Anchors

##### 6.2.8.2.1	Overview

Table 6.2.8.2.1-1 provides an overview of the H.264/AVC anchor tuples. Keys are identified to refer to the anchors in the context of the scenario.

Editor’s Note: This is not yet complete.

Table 6.2.8.2.1-1 Anchor Tuple generation with H.264/AVC for Full HD Scenario

| Key | Clause | Reference Sequence | Reference Encoder | Configuration | Variations | Anchor Key |
| --- | --- | --- | --- | --- | --- | --- |
| S1-A1-264 | 6.2.8.2.2 | S1-R1 | JM19.0 | JM-01 | QP = [17, 20, 23, 26, 29, 32] | S1-A1-264-<QP> |
| S1-A2-264 | 6.2.8.2.2 | S1-R2 | JM19.0 | JM-01 | QP = [17, 20, 23, 26, 29, 32] | S1-A2-264-<QP> |
| S1-A3-264 | 6.2.8.2.2 | S1-R3 | JM19.0 | JM-01 | QP = [17, 20, 23, 26, 29, 32] | S1-A2-264-<QP> |

##### 6.2.8.2.2	JM-01

This anchor tuple produces an anchor over 60 seconds with a typical configuration of H.264/AVC:

-	Profile H.264/AVC Progressive High-Profile Level 4.2 [7]

-	Random access and switching at 1 second interval

-	Fixed QP settings

-	IBBP GOP structures

Different sequences are used with resolutions 1080p, 900p and 720p.

#### 6.2.8.3	H.265/HEVC Anchors

##### 6.2.8.3.1	Overview

Table 6.2.8.3.1-1 provides an overview of the H.265/HEVC anchor tuples. Keys are identified to refer to the anchors in the context of the scenario.

Editor’s Note: This is not yet complete.

Table 6.2.8.3.1-1 Anchor Tuple generation with H.265/HEVC for Full HD Scenario

| Key | Clause | Reference Sequence | Reference Encoder | Configuration | Variations | Anchor Key |
| --- | --- | --- | --- | --- | --- | --- |

| S1-A1-265 | 6.2.8.3.2 | S1-R1 | HM16.22 | HM-0x | QP = [22, 25, 28, 31, 34, 37] | S1-A1-265-<QP> |
| --- | --- | --- | --- | --- | --- | --- |

.8.3.1-1 Anchor Tuple generation with H.265/HEVC for Full HD Scenario

| Key | Clause | Reference Sequence | Reference Encoder | Configuration | Variations | Anchor Key |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

##### 6.2.8.3.2	HM-0x

tbd

### 6.2.9	Anchor Results

tbd

### 6.2.10	Additional Information and Performance Data

Tbd

## 6.3	Scenario 2: 4K-TV

### 6.3.1	Motivation

Streaming towards mobile devices is undoubtly the first natural use-case expected for 5G-media streaming. However, consumption of video services on fixed receivers (e.g. TV sets) remains a preferable way of experiencing the high-quality content, whether it is for on-demand (e.g., blockbuster movies) or live services (e.g., sport events). Recent reports from largest VOD platforms such as Netflix confirm that assumption and show that the primary way of watching content remains fixed TV screens, covering 70% of devices 6 months after subscription [17]. In the same way, YouTube indicates that service usage on fixed TV set remains an inevitable way of accessing the content, with 250M of hours viewed per day on TV screens [18]. As 5G media streaming targets a wide range of connected devices and should be able to deliver video streams to many compatible high-resolution receivers, (e.g. 5G-HDMI-sticks, 5G-StB/5G-MediaGateway or even 5G-TV sets) the inclusion of 4K TV Scenarios for 5G Video codec evaluation is important.

First, the 4K-TV set is currently the most established way of displaying premium quality services using latest technology improvements for video content, including High-Dynamic-Range (HDR) and Wide-Colour-Gamut (WCG). Latest statistics from the Ultra-HD forum indicate that 148 UHD services are currently on-air, 74% being linear, 45% of those using HDR [19]. In addition, a large number of SVOD operators propose 4K access in their subscription packages (e.g. Netflix™ and Amazon Prime™). All these services may eventually take advantage of 5G-network capabilities to increase the device reach and enlarge audiences. This scenario is also endorsed by strong shipment forecasts, as indicated in the latest IHS 4K-TV UHD bluebook [20].

### 6.3.2	Description of the Anticipated Application

In the context of 3GPP services, 5G Media Streaming [13] as well as the TV Video Profiles [3] are specifications addressing this 4K-TV scenario. Both, 5G Media Streaming [13] and TV Video Profiles [3] build on CMAF-based Segment formats and DASH distribution. From 3GPP TS 26.116, the following operation points may be considered in scope of the 4K-TV Streaming Scenario (pending availability of appropriate test content):

-	H.265/HEVC UHD, see 3GPP TS 26.116 [3] clause 4.5.4.

-	H.265/HEVC UHD HDR, see 3GPP TS 26.116 [3] clause 4.5.6.

-	H.265/HEVC UHD HDR HLG, see 3GPP TS 26.116 [3] clause 4.5.8.

This scenario is based on CMAF (including LL-DASH and HLS-LL) distribution of UHD-TV video services over 5G networks to 5G/non-5G capable devices. This includes 5G-equipped devices (e.g. smartphone, tablets, …) but also other devices gateway (e.g. TV sets, HDMI-Stick…) accessing services through a “5G-gateway” which can be a mobile phone or a home gateway. As multiple linear services will be delivered in parallel (news, sport, talk show…) in a similar manner as traditional TV services in a multiplex (potentially using multicast/broadcast delivery over 5G). In certain environments, High Frame Rate (HFR) beyond 60 fps is considered, e.g. in DVB and ATSC broadcast specifications. 3GPP does not have any HFR TV video profiles yet.

Important aspects that are expected to be considered when evaluating a codec in the context of this 4K-TV scenario are:

-	Quality and Coding Efficiency:

-	High and uninterrupted visual quality, considering the service constraints.

-	Any savings can provide significant benefits due to the expected large volume of the traffic either in quality or network utilization.

- 	Adaptive Bitrate streaming:

-	Multiple bit rates are provided, typically with a ladder of 30–50% to permit bandwidth adaptation. The use of constant bit rate (CBR) encoding maximises reuse of a common ladder of encoded representations across multiple distribution networks. The use of capped variable bit rate (VBR) encoding allows the bit rate to be varied according to the difficulty of the source material while maintaining the ability to distribute the encoded representations through distribution networks with fixed capacity. This also maximises the usage of a common ladder across multiple distribution networks.

-	CMAF Fragments of size typically in the range of 1–6s to permit seamless switching for bit rate adaptation.

-	Regular Random Access, typically every 1–2 seconds according to 3GPP TS 26.116 [3]. To achieve clean switching in both sound and picture when moving between different encoded representations in the ladder, 3.84 seconds enables video segment boundaries to be aligned with an integer number of audio Access Units, if a 50fps video signal and 48kHz audio signal is used.

-	Encoding in this scenario is typically done as

-	Live and On-Demand distribution and encoding

- 	Server and Cloud-based Encoding

-	Capable of encoding multiple services at variable bitrate, inside a fixed dedicated resource (statistical multiplexing).

### 6.3.3	Source Format Properties

Table 6.3.3-1 provides an overview of the different source signal properties for 4K-TV. This information is used to select proper test sequences.

Table 6.3.3-1 4K-TV source format properties

| Source format properties | 4K-TV |
| --- | --- |
| Spatial resolution |  | 3840 x 2160 (Permitted distribution formats: 2560 × 1440, 1920 × 1080, 1600 × 900, 1280 × 720) |  |
| Chroma format |  | Y’CbCr |  |
| Chroma subsampling |  | 4:2:0 |  |
| Picture aspec ratio |  | 16:9 |  |
| Frame rates |  | 24; 50; 60; 24/1.001; 60/1.001; [100; 120] Hz |  |
| Bit depth |  | 10 |  |
| Colour space formats |  | BT.2020 [15] |  |
| Transfer characteristics |  | BT.2020 [15], BT.2100 [16] (PQ & HLG) |  |

NOTE:	High Frame Rate (HFR) is not supported by 3GPP TV Video profiles defined in 3GPP TS 26.116 [3] in release 16. However, HFR is introduced in this clause for consideration on the video codec performances.

### 6.3.4	Encoding and Decoding Constraints

Table 6.3.4-1 provides an overview of encoding and decoding constraints for 4K-TV category using legacy codec HEVC. This will support the definition of detailed test conditions. It is noted that no relevant profiles exist in TS26.116 and TS26.511 for HFR 4K-TV content.

Table 6.3.4-1 Encoding and Decoding Configurations for 4K-TV with legacy HEVC codec

| Encoding and Decoding Constraints | H.265/HEVC 4K-TV | H.265/HEVC 4K-TV HFR |
| --- | --- | --- |
| Relevant Codec and Codec Profile/Levels according to TS26.116 and TS26.511. | H.265/HEVC Main-10 Profile Level 5.1 [8] | No relevant 3GPP profiles, should be aligned with H.265/HEVC Main-10 Profile Level 5.2 [8] |
| RAP period | 3.84sec, 1sec | 3.84sec, 1sec |
| Bit rate parameters (CBR, VBR, CAE, HRD parameters) | QP = [tbd] B = {10,20,30,40} Mbps [49] CBR and capped-VBR | QP = [tbd] B = {10,20,30,40} Mbps [49] CBR and capped-VBR |
| Latency requirements and specific encoding settings | No latency requirements beyond RAP so picture reordering allowed | No latency requirements beyond RAP so picture reordering allowed |
| Encoding complexity context | real-time encoding, cloud-based encoding, offline encoding, etc. | real-time encoding, cloud-based encoding, offline encoding, etc. |
| Required decoding capabilities | H.265/HEVC Main-10 Profile Level 5.1 [8] | H.265/HEVC Main-10 Profile Level 5.2 [8] |

### 6.3.5	Performance Metrics

Performance is assessed using BD-Rate computation, with PSNR, SSIM and VMAF metrics as objective quality criterion. Regarding complexity considerations, encoding/decoding runtime is provided.

### 6.3.6	Interoperability Considerations

In order to use a codec in the context of 5G Media Streaming services in 3GPP TS 26.511 [13] and for TV Video profiles in 3GPP TS 26.116 [3], the same considerations for interoperability as for FullHD according to clause 6.2.6 apply.

For additional details, please refer to 3GPP TS 26.116 [3] and 3GPP TS 26.511 [13].

### 6.3.7	Reference Sequences

Table 6.3.7-1 provides the selected reference sequences for this scenario for SDR and Table 6.3.7-2 provides the selected reference sequences for this scenario for HDR. Keys are defined to refer to the sequences in the context of the scenario. The sequences are named and a reference to the details of the sequence is provided. Annex C.3 describes in details the selection process conducted to build the test sequences considered for this scenario, the outcome of this process is reported in the table below.

Table 6.3.7-1 SDR Reference Sequences for 4K-TV scenario

| Key | Name | Reference | Resolution | Frame rate | Colour Gamut | Number of Frames | Scene Cut |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S2-R1 | Brest-Sedof | Annex C.3.1.3.1 | 3840 x 2160 | 60 | BT.2020 | 600 | 0 |
| S2-R2 | Rain Fruits | Annex C.3.1.3.2 | 3840 x 2160 | 50 | BT.709 | 600 | 0 |
| S2-R3 | Park Joy | Annex C.3.1.3.3 | 3840 x 2160 | 50 | BT.709 | 500 | 0 |
| S2-R4 | Soccer | Annex C.3.1.3.4 | 3840 x 2160 | 23.98 | BT.709 | 408 | 4 |
| S2-R5 | Tunnel Flag | Annex C.3.1.3.5 | 4096 x 2160 | 59.94 | BT.709 | 6001199 | 0 |
| S2-R6 | Boat | Annex C.3.1.3.6 | 4096 x 2160 | 59.94 | BT.709 | 300 | 0 |
| S2-R7 | Fountain | Annex C.3.1.3.7 | 4096 x 2160 | 59.94 | BT.709 | 1199 | 0 |
| S2-R8 | Riverbank | Annex C.3.1.3.8 | 3840 x 2160 | 50 | BT.709 | 600 | 0 |

Table 6.3.7-2 HDR Reference Sequences for 4K-TV scenario

| Key | Name | Reference | Resolution | Frame rate | Colour Gamut | Number of Frames | Scene Cut |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S2-R11 | Life-Untouched | Annex C.3.2.3.1 | 4096 x 2160 | 59.94 | BT.2020 | 480 | 0 |
| S2-R12 | Meridian | Annex C.3.2.3.2 | 3840 x 2160 | 59.94 | BT.2020 | 324 | 0 |
| S2-R13 | Sol-Levante | Annex C.3.2.3.3 | 3840 x 2160 | 24 | BT.2020 | 144 | 0 |
| S2-R14 | Cosmos | Annex C.3.2.3.4 | 3840 x 2160 | 24 | BT.2020 | 181 | 0 |
| S2-R15 | Elevator | Annex C.3.2.3.5 | 4096 x 2160 | 59.94 | BT.2020 | 431 | 0 |
| S2-R16 | Sparks | Annex C.3.2.3.6 | 4096 x 2160 | 59.94 | BT.2020 | 260 | 0 |
| S2-R17 | Nocturne | Annex C.3.2.3.7 | 3840 x 2160 | 60 | BT.2020 | 369 | 0 |

### 6.3.8	Anchor Definition

#### 6.3.8.1	Overview

First, the legacy codec HEVC is tested to assess the relevance of what’s already in the 3GPP specification for this particular 4K-TV scenario. For this first test, two HEVC implementations are compared (x265 and reference software HM), according to encoding constraints derived from Table 6.3-2. For x265, encoding presets are selected to cover the desired encoding complexity contexts, for live and offline encoding. The tested rate-control modes are CBR and capped-VBR to fit with the possible delivery methods (single or multiple services inside a resource). In this first test, the HEVC reference implementation HM is evaluated with CBR rate-control on, at coding-tree-unit (CTU) granularity. In addition, fixed QP encoding is also carried out to provide additional anchor points aligned with formal MPEG/JVET common test conditions (potentially for future comparison with other codecs, if needed).

#### 6.3.8.2	H.264/AVC Anchors

No H.264/AVC Anchors are defined for this scenario.

#### 6.3.8.3	H.265/HEVC Anchors

##### 6.3.8.3.1	Overview

Table 6.3.8.3.1-1 provides an overview of the H.265/HEVC anchor tuples. Keys are identified to refer to the anchors in the context of the scenario.

First, the legacy codec HEVC is tested to assess the relevance of what’s already in the 3GPP specification for this particular 4K-TV scenario. For this first test, two HEVC implementations are compared (x265 and reference software HM), according to encoding constraints derived from Table 6.3-2. For x265, encoding presets are selected to cover the desired encoding complexity contexts, for live and offline encoding. The tested rate-control modes are CBR and capped-VBR to fit with the possible delivery methods (single or multiple services inside a resource). In this first test, the HEVC reference implementation HM is evaluated with CBR rate-control on, at coding-tree-unit (CTU) granularity. In addition, fixed QP encoding is also carried out to provide additional anchor points aligned with formal MPEG/JVET common test conditions (potentially for future comparison with other codecs, if needed).

Table 6.2.8.3.1-1 Anchor Tuple generation with H.265/HEVC for Full HD Scenario

| Key | Clause | Reference Sequence | Reference Encoder | Configuration | Variations | Anchor Key |
| --- | --- | --- | --- | --- | --- | --- |

Table 6.3.8.3.1-1 Anchor Tuple generation with H.265/HEVC for Full HD Scenario

| Key | Clause | Reference Sequence | Reference Encoder | Configuration | Variations | Anchor Key |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S2-A1-265 | 6.3.8.3.2 | S2-R1 | HM16.22 | HM-0x | QP = [22, 25, 28, 31, 34, 37] | S1-A1-265-<QP> |  |

##### 6.3.8.3.2	HM-0x

As reference software for HEVC, the following is used

-	HM16.22 https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.22/

Example setting: https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.22/cfg/encoder_randomaccess_main10.cfg with following proposed changes

- 	IntraPeriod: Intra Period such that 1 second is achieved

- 	DecodingRefreshType: 1 (CRA) changed to 2 (IDR)

- 	GOPSize: adjusted to Intra

-	QP = [22, 25, 28, 31, 34, 37]

### 6.3.9	Anchor Results

tbd

### 6.3.10	Additional Information and Performance Data

Tbd (with references).

## 6.4	Scenario 3: Screen Content Scenario

### 6.4.1	Motivation

This scenario mostly motivates cases for which content goes beyond videographic content, and in particular includes computer generated imagery (CGI). Several application spaces rely on screen content types such as videoconferencing and webinars with presentation slides displayed, or remote desktop applications for demos or remote assistance. These services include massive online videoconferencing systems and telepresence systems both being particularly popular with remote collaboration.

For telepresence with screen-sharing applications, some information related to video is collected in the following:

-	MS Teams™ [26] as of end of 2019.

o	There are several formats supported for video. Two key properties of a video format are its frame size and color format. Supported frame sizes include 640x360 ("360p"), 1280x720 ("720p"), and 1920x1080 ("1080p"). Supported color formats include NV12 (12 bits per pixel) and RGB24 (24 bits per pixel).

o	A "720p" video frame contains 921,600 pixels (1280 times 720). In the RGB24 color format, each pixel is represented as 3 bytes (24-bits) comprised of one byte each of red, green, and blue color components. Therefore, a single 720p RGB24 video frame requires 2,764,800 bytes of data (921,600 pixels times 3 bytes/pixel). At a frame rate of 30fps, sending 720p RGB24 video frames means processing approximately 80 MB/s of content (which is substantially compressed by the H.264 video codec before network transmission).

-	Other tools are for further study.

As an example of hardware implementation, the Intel Xe LP™ chipset supports HEVC screen content profile (see slide 101 of [41]).

### 6.4.2	Description of the Anticipated Application

3GPP until now has very restricted set of services but based on the considerations in clause 6.4.1, the following encoding benchmark capabilities are considered for decoding:

-	H.264 (AVCHD) YUV 4:2:0, YUV 4:4:4, 8 bit, Max Resolution 1920x1080

-	H.265 (HEVC) YUV 4:2:0, YUV 4:4:4, 10 bit, Max Resolution 4096 x 2048

The considered scenario is low-latency streaming or conversational. Important aspects that are expected to be considered when evaluating a codec in the context of this:

-	Quality and Coding Efficiency:

-	The ability to compress computer-generated content. Typically, it means the ability to have non perceptible intra refreshes and the ability to maintain stability on low frequency areas (such as uniform backgrounds) as well as maintaining details on high frequencies (particularly for text)

-	The ability compress YUV 4:2:0 and 4:4:4 content.

- 	Considered settings for encoding:

-	Low-latency settings

-	No specific error resilience mechanisms

-	Encoding in this scenario is typically done as

-	Real-time encoding

### 6.4.3	Source Format Properties

Table 6.4.3-1 provides an overview of the different source signal properties for Screen Content Sharing. This information is used to select proper test sequences.

Table 6.4.3-1 Screen Content Sharing source properties

| Source format properties | Screen Content |
| --- | --- |
| Spatial resolution |  | 1920 x 1080 |  |
| Chroma format |  | Y’CbCr, RGB |  |
| Chroma subsampling |  | 4:2:0, 4:4:4 |  |
| Picture aspect ratio |  | 16:9 |  |
| Frame rates |  | 25, 30, 50, 60 Hz |  |
| Bit depth |  | 8 |  |
| Colour space formats |  | BT.709 |  |
| Transfer characteristics |  | BT.709 |  |

### 6.4.4	Encoding and Decoding Constraints

Table 6.4.4-1 provides an overview of the different codec tools per profile that may be suitable for coding screen content sequences.

Table 6.4.4-1 Screen Content Tools per Profile

| Screen content tools | AVC | HEVC |
| --- | --- | --- |
| main profile | Not applicable | Transform skip |
| range extension profile | not applicable | Residual Differential Pulse Code Modulation (RDPCM) (implicit intra/explicit inter), |
| screen content profile | Not applicable | Intra Block Copy (full frame or less), Palette, Adaptive Color Transform |

Table 6.4.4-2 provides an overview of encoding and decoding constraints for Screen Content scenario using AVC and HEVC codecs operating points. This will support the definition of detailed test conditions.

Table 6.4.4-2 Encoding and Decoding Configurations for Screen Content

| Encoding and Decoding Constraints | H.264/AVC Full HD | H.265/HEVC Full HD |
| --- | --- | --- |
| Relevant Codec and Codec Profile/Levels according to TS26.116 and TS26.511. | H.264/AVC Progressive High Profile Level 4.0 [7] | H.265/HEVC Main-10 Profile H.265/HEVC Screen Content profile Level 4.1, Level 5.1, Level 6.1 |
| Random access frequency | 1 second, infinite | 1 second, infinite |
| Error resiliency requirements | Fixed QP | Fixed QP |
| Bit rates and quality configuration | Low-latency requirements, Low-delay configuration | Low-latency requirements, Low-delay B, low-delay-P |
| Bit rate parameters (CBR, VBR, CAE, HRD parameters) | real-time encoding. | real-time encoding. |
| ABR encoding requirements (switching frequency, etc.) | H.264/AVC Main Profile Level 4.0 [7] | H.265/HEVC Main-10 Profile H.265/HEVC Screen Content profile Level 4.1, 5.1, 6.1 [8] |
| Latency requirements and specific encoding settings | H.264/AVC Main Profile Level 4.0 [7] | H.265/HEVC Main-10 Profile H.265/HEVC Screen Content profile Level 4.1, Level 5.1, Level 6.1 |
| Encoding complexity context | 1 second, infinite | 1 second, infinite |
| Required decoding capabilities | Fixed QP | Fixed QP |

### 6.4.5	Performance Metrics

All SDR Metrics as defined in clause 5.5.2 apply.

Editor’s Note: We leave the decision on characterization open, which metrics are to be used

### 6.4.6	Interoperability Considerations

For screen content, RTP-based communication is expected.

### 6.4.7	Reference Sequences

Table 6.4.7-1 provides the selected reference sequences for this scenario. Keys are defined to refer to the sequences in the context of the scenario. The sequences are named and a reference to the details of the sequence is provided. A justification is provided, why this sequence is selected.

Reference sequences illustrating the screen content scenario are described in Annex C. They contain either synthetic content from a presentation such as a slide deck with text and graphics.

The details are also provided here: http://dash.akamaized.net/WAVE/3GPP/5GVideo/Anchors/Scenario-3/reference-sequence.csv.

Table 6.4.7-1 Reference Sequences for Screen Content Scenario

| Key | Name | Reference | Justification/Comment |
| --- | --- | --- | --- |
| S3-R01 | MovingText2-4K-10bit | Annex C.6.2 |  |
| S3-R02 | MovingText2-4K-8bit | Annex C.6.2 |  |
| S3-R03 | MovingText2-FullHD-10bit | Annex C.6.2 |  |
| S3-R04 | MovingText2-FullHD-8bit | Annex C.6.2 |  |
| S3-R05 | TextMixTransitions-4K-10bit | Annex C.6.3 |  |
| S3-R06 | TextMixTransitions-4K-8bit | Annex C.6.3 |  |
| S3-R07 | TextMixTransitions-FullHD-10bit | Annex C.6.3 |  |
| S3-R08 | TextMixTransitions-FullHD-8bit | Annex C.6.3 |  |
| S3-R09 | GraphicsMixSimple-4K-10bit | Annex C.6.4 |  |
| S3-R10 | GraphicsMixSimple-4K-8bit | Annex C.6.4 |  |
| S3-R11 | GraphicsMixSimple-FullHD-10bit | Annex C.6.4 |  |
| S3-R12 | GraphicsMixSimple-FullHD-8bit | Annex C.6.4 |  |
| S3-R13 | GraphicsMixTransitions-4K-10bit | Annex C.6.5 |  |
| S3-R14 | GraphicsMixTransitions-4K-8bit | Annex C.6.5 |  |
| S3-R15 | GraphicsMixTransitions-FullHD-10bit | Annex C.6.5 |  |
| S3-R16 | GraphicsMixTransitions-FullHD-8bit | Annex C.6.5 |  |

### 6.4.8	Anchor Definition

#### 6.4.8.1	Overview

This clause provides details on how to generate the anchors for the Screen Content scenario.

The screen content configuration relies on low delay encoding modes with the optional use of Screen Content encoding tools when possible (for HEVC).

#### 6.4.8.2	H.264/AVC Anchors

##### 6.4.8.2.1	Overview

Table 6.4.8.2.1-1 provides an overview of the H.264/AVC anchor tuples. Keys are identified to refer to the anchors in the context of the scenario.

Editor’s Note: This is not yet complete.

Table 6.4.8.2.1-1 Anchor Tuple generation with H.264/AVC for Screen Content Scenario

| Key | Clause | Reference Sequence | Reference Encoder | Config | Variations | Anchor Key |
| --- | --- | --- | --- | --- | --- | --- |
| S3-A1-264 | 6.4.8.2.2 | S3-R1 | JM19.0 | JM-0x | QP = [17, 20, 23, 26, 29, 32] | S3-A1-264-<QP> |
| S3-A2-264 | 6.4.8.2.2 | S3-R2 | JM19.0 | JM-0x | QP = [17, 20, 23, 26, 29, 32] | S3-A2-264-<QP> |
| S3-A3-264 | 6.4.8.2.2 | S3-R3 | JM19.0 | JM-0x | QP = [17, 20, 23, 26, 29, 32] | S3-A2-264-<QP> |

##### 6.4.8.2.2	JM-0x

The encoder configuration file is:

-	https://vc-git.hhi.fraunhofer.de/jct-vc/JM/-/blob/master/cfg/encoder_main.cfg

With the following adaptations:

-	ProfileIDC = XX (High Profile)

-	LevelIDC = 42 (for 1080p60) : 52 (for 4Kp60)

-	IntraPeriod = 0 (only first) or 64 (approaching 1s for 60fps)

-	IDRPeriod = [Same as IntraPeriod]

-	QPISlice = QPPSlice = [17, 22, 27, 32, 37, 42]

-	SearchRange = 64 for fullHD ; 128 for 4K

-	NumberOfReferenceFrames = 2

-	NumberBFrames = 0 [TBC]

-	SearchMode = 0 (fast full search)

#### 6.4.8.3	H.265/HEVC Anchors

##### 6.4.8.3.1	Overview

Table 6.4.8.3.1-1 provides an overview of the H.265/HEVC anchor tuples. Keys are identified to refer to the anchors in the context of the scenario.

The details are also provided here: http://dash.akamaized.net/WAVE/3GPP/5GVideo/Anchors/Scenario-3/anchors.csv.

Table 6.4.8.3.1-1 Anchor Tuple generation with H.265/HEVC for Screen Content Scenario

| Key | Clause | Reference Sequence | Reference Encoder | Configuration | Variations | Anchor Key |
| --- | --- | --- | --- | --- | --- | --- |
| S3-A01-265 | 6.4.8.3.2 | S3-R01 | HM16.22 | HM-01 | QP = [22, 27, 32, 37] | S3-A01-265-<QP> |
| S3-A02-265 | 6.4.8.3.2 | S3-R02 | HM16.22SCM8.8 | HM-01 | QP = [22, 27, 32, 37] | S3-A02-265-<QP> |
| S3-A03-265 | 6.4.8.3.2 | S3-R03 | HM16.22 | HM-01 | QP = [22, 27, 32, 37] | S3-A03-265-<QP> |
| S3-A04-265 | 6.4.8.3.2 | S3-R04 | HM16.22 | HM-01 | QP = [22, 27, 32, 37] | S3-A04-265-<QP> |
| S3-A05-265 | 6.4.8.3.2 | S3-R05 | HM16.22 | HM-01 | QP = [22, 27, 32, 37] | S3-A05-265-<QP> |
| S3-A06-265 | 6.4.8.3.2 | S3-R06 | HM16.22 | HM-01 | QP = [22, 27, 32, 37] | S3-A06-265-<QP> |
| S3-A07-265 | 6.4.8.3.2 | S3-R07 | HM16.22 | HM-01 | QP = [22, 27, 32, 37] | S3-A07-265-<QP> |
| S3-A08-265 | 6.4.8.3.2 | S3-R08 | HM16.22 | HM-01 | QP = [22, 27, 32, 37] | S3-A08-265-<QP> |
| S3-A09-265 | 6.4.8.3.2 | S3-R09 | HM16.22 | HM-01 | QP = [22, 27, 32, 37] | S3-A09-265-<QP> |
| S3-A10-265 | 6.4.8.3.2 | S3-R10 | HM16.22 | HM-01 | QP = [22, 27, 32, 37] | S3-A10-265-<QP> |
| S3-A11-265 | 6.4.8.3.2 | S3-R11 | HM16.22 | HM-01 | QP = [22, 27, 32, 37] | S3-A11-265-<QP> |
| S3-A12-265 | 6.4.8.3.2 | S3-R12 | HM16.22 | HM-01 | QP = [22, 27, 32, 37] | S3-A12-265-<QP> |
| S3-A13-265 | 6.4.8.3.2 | S3-R13 | HM16.22 | HM-01 | QP = [22, 27, 32, 37] | S3-A13-265-<QP> |
| S3-A14-265 | 6.4.8.3.2 | S3-R14 | HM16.22 | HM-01 | QP = [22, 27, 32, 37] | S3-A14-265-<QP> |
| S3-A15-265 | 6.4.8.3.2 | S3-R15 | HM16.22 | HM-01 | QP = [22, 27, 32, 37] | S3-A15-265-<QP> |
| S3-A16-265 | 6.4.8.3.2 | S3-R16 | HM16.22 | HM-01 | QP = [22, 27, 32, 37] | S3-A16-265-<QP> |
| S3-A17-265 | 6.4.8.3.3 | S3-R01 | HM16.22 | HM-02 | QP = [22, 27, 32, 37] | S3-A17-265-<QP> |
| S3-A18-265 | 6.4.8.3.3 | S3-R02 | HM16.22 | HM-02 | QP = [22, 27, 32, 37] | S3-A18-265-<QP> |
| S3-A19-265 | 6.4.8.3.3 | S3-R03 | HM16.22 | HM-02 | QP = [22, 27, 32, 37] | S3-A19-265-<QP> |
| S3-A20-265 | 6.4.8.3.3 | S3-R04 | HM16.22 | HM-02 | QP = [22, 27, 32, 37] | S3-A20-265-<QP> |
| S3-A21-265 | 6.4.8.3.3 | S3-R05 | HM16.22 | HM-02 | QP = [22, 27, 32, 37] | S3-A21-265-<QP> |
| S3-A22-265 | 6.4.8.3.3 | S3-R06 | HM16.22 | HM-02 | QP = [22, 27, 32, 37] | S3-A22-265-<QP> |
| S3-A23-265 | 6.4.8.3.3 | S3-R07 | HM16.22 | HM-02 | QP = [22, 27, 32, 37] | S3-A23-265-<QP> |
| S3-A24-265 | 6.4.8.3.3 | S3-R08 | HM16.22 | HM-02 | QP = [22, 27, 32, 37] | S3-A24-265-<QP> |
| S3-A25-265 | 6.4.8.3.3 | S3-R09 | HM16.22 | HM-02 | QP = [22, 27, 32, 37] | S3-A25-265-<QP> |
| S3-A26-265 | 6.4.8.3.3 | S3-R10 | HM16.22 | HM-02 | QP = [22, 27, 32, 37] | S3-A26-265-<QP> |
| S3-A27-265 | 6.4.8.3.3 | S3-R11 | HM16.22 | HM-02 | QP = [22, 27, 32, 37] | S3-A27-265-<QP> |
| S3-A28-265 | 6.4.8.3.3 | S3-R12 | HM16.22 | HM-02 | QP = [22, 27, 32, 37] | S3-A28-265-<QP> |
| S3-A29-265 | 6.4.8.3.3 | S3-R13 | HM16.22 | HM-02 | QP = [22, 27, 32, 37] | S3-A29-265-<QP> |
| S3-A30-265 | 6.4.8.3.3 | S3-R14 | HM16.22 | HM-02 | QP = [22, 27, 32, 37] | S3-A30-265-<QP> |
| S3-A31-265 | 6.4.8.3.3 | S3-R15 | HM16.22 | HM-02 | QP = [22, 27, 32, 37] | S3-A31-265-<QP> |
| S3-A32-265 | 6.4.8.3.3 | S3-R16 | HM16.22 | HM-02 | QP = [22, 27, 32, 37] | S3-A32-265-<QP> |
| S3-A33-265 | 6.4.8.3.4 | S3-R01 | SCM8.8 | SCC-01 | QP = [22, 27, 32, 37] | S3-A33-265-<QP> |
| S3-A34-265 | 6.4.8.3.4 | S3-R02 | SCM8.8 | SCC-01 | QP = [22, 27, 32, 37] | S3-A34-265-<QP> |
| S3-A35-265 | 6.4.8.3.4 | S3-R03 | SCM8.8 | SCC-01 | QP = [22, 27, 32, 37] | S3-A35-265-<QP> |
| S3-A36-265 | 6.4.8.3.4 | S3-R04 | SCM8.8 | SCC-01 | QP = [22, 27, 32, 37] | S3-A36-265-<QP> |
| S3-A37-265 | 6.4.8.3.4 | S3-R05 | SCM8.8 | SCC-01 | QP = [22, 27, 32, 37] | S3-A37-265-<QP> |
| S3-A38-265 | 6.4.8.3.4 | S3-R06 | SCM8.8 | SCC-01 | QP = [22, 27, 32, 37] | S3-A38-265-<QP> |
| S3-A39-265 | 6.4.8.3.4 | S3-R07 | SCM8.8 | SCC-01 | QP = [22, 27, 32, 37] | S3-A39-265-<QP> |
| S3-A40-265 | 6.4.8.3.4 | S3-R08 | SCM8.8 | SCC-01 | QP = [22, 27, 32, 37] | S3-A40-265-<QP> |
| S3-A41-265 | 6.4.8.3.4 | S3-R09 | SCM8.8 | SCC-01 | QP = [22, 27, 32, 37] | S3-A41-265-<QP> |
| S3-A42-265 | 6.4.8.3.4 | S3-R10 | SCM8.8 | SCC-01 | QP = [22, 27, 32, 37] | S3-A42-265-<QP> |
| S3-A43-265 | 6.4.8.3.4 | S3-R11 | SCM8.8 | SCC-01 | QP = [22, 27, 32, 37] | S3-A43-265-<QP> |
| S3-A44-265 | 6.4.8.3.4 | S3-R12 | SCM8.8 | SCC-01 | QP = [22, 27, 32, 37] | S3-A44-265-<QP> |
| S3-A45-265 | 6.4.8.3.4 | S3-R13 | SCM8.8 | SCC-01 | QP = [22, 27, 32, 37] | S3-A45-265-<QP> |
| S3-A46-265 | 6.4.8.3.4 | S3-R14 | SCM8.8 | SCC-01 | QP = [22, 27, 32, 37] | S3-A46-265-<QP> |
| S3-A47-265 | 6.4.8.3.4 | S3-R15 | SCM8.8 | SCC-01 | QP = [22, 27, 32, 37] | S3-A47-265-<QP> |
| S3-A48-265 | 6.4.8.3.4 | S3-R16 | SCM8.8 | SCC-01 | QP = [22, 27, 32, 37] | S3-A48-265-<QP> |
| S3-A49-265 | 6.4.8.3.5 | S3-R01 | SCM8.8 | SCC-02 | QP = [22, 27, 32, 37] | S3-A49-265-<QP> |
| S3-A50-265 | 6.4.8.3.5 | S3-R02 | SCM8.8 | SCC-02 | QP = [22, 27, 32, 37] | S3-A50-265-<QP> |
| S3-A51-265 | 6.4.8.3.5 | S3-R03 | SCM8.8 | SCC-02 | QP = [22, 27, 32, 37] | S3-A51-265-<QP> |
| S3-A52-265 | 6.4.8.3.5 | S3-R04 | SCM8.8 | SCC-02 | QP = [22, 27, 32, 37] | S3-A52-265-<QP> |
| S3-A53-265 | 6.4.8.3.5 | S3-R05 | SCM8.8 | SCC-02 | QP = [22, 27, 32, 37] | S3-A53-265-<QP> |
| S3-A54-265 | 6.4.8.3.5 | S3-R06 | SCM8.8 | SCC-02 | QP = [22, 27, 32, 37] | S3-A54-265-<QP> |
| S3-A55-265 | 6.4.8.3.5 | S3-R07 | SCM8.8 | SCC-02 | QP = [22, 27, 32, 37] | S3-A55-265-<QP> |
| S3-A56-265 | 6.4.8.3.5 | S3-R08 | SCM8.8 | SCC-02 | QP = [22, 27, 32, 37] | S3-A56-265-<QP> |
| S3-A57-265 | 6.4.8.3.5 | S3-R09 | SCM8.8 | SCC-02 | QP = [22, 27, 32, 37] | S3-A57-265-<QP> |
| S3-A58-265 | 6.4.8.3.5 | S3-R10 | SCM8.8 | SCC-02 | QP = [22, 27, 32, 37] | S3-A58-265-<QP> |
| S3-A59-265 | 6.4.8.3.5 | S3-R11 | SCM8.8 | SCC-02 | QP = [22, 27, 32, 37] | S3-A59-265-<QP> |
| S3-A60-265 | 6.4.8.3.5 | S3-R12 | SCM8.8 | SCC-02 | QP = [22, 27, 32, 37] | S3-A60-265-<QP> |
| S3-A61-265 | 6.4.8.3.5 | S3-R13 | SCM8.8 | SCC-02 | QP = [22, 27, 32, 37] | S3-A61-265-<QP> |
| S3-A62-265 | 6.4.8.3.5 | S3-R14 | SCM8.8 | SCC-02 | QP = [22, 27, 32, 37] | S3-A62-265-<QP> |
| S3-A63-265 | 6.4.8.3.5 | S3-R15 | SCM8.8 | SCC-02 | QP = [22, 27, 32, 37] | S3-A63-265-<QP> |
| S3-A64-265 | 6.4.8.3.5 | S3-R16 | SCM8.8 | SCC-02 | QP = [22, 27, 32, 37] | S3-A64-265-<QP> |

##### 6.4.8.3.2	HM-01: Main 10 Profile with no fixed Intra

To generate the anchor bitstreams, HM16.22 is used.

The settings are defined

in the attached configuration file hm-01.cfg.

In summary, each source sequence is encoded with the following changes:

-	QP: [22, 27, 32, 37, 42]

-	IntraPeriod with no fix interval

-	GOPSize is equal to 8. Each P picture refers to up to 4 preceding pictures in display order

-	IntraQPOffset is -1. P picture QP offsets are alternatively equal to 4, 5, and are set to 1 every 8 pictures.

##### 6.4.8.3.3	HM-02: Main 10 Profile with fixed Intra every second

To generate the anchor bitstreams, HM16.22 is used.

The settings are defined in the attached configuration file hm-02.cfg.

In summary, each source sequence is encoded with the following changes:

-	QP: [22, 27, 32, 37, 42]

-	IntraPeriod such that 1 second is achieved

-	DecodingRefreshType: (2) IDR

-	IntraQPOffset and QPoffsets are set equal to 0

-	Each P picture refers to immediately preceding pictures in display order.

##### 6.4.8.3.4	SCC-01: Screen Content Profile with no fixed Intra

To generate the anchor

bitstreams SCM-8.8 is used.

The settings are defined

in the attached configuration file scc-01.cfg.

In summary, each source sequence is encoded with

-	QP: [22, 27, 32, 37, 42]

-	IntraPeriod with no fix interval

-  	GOPSize is equal to 8. Each P picture refers to up to 4 preceding pictures in display order

-  	IntraQPOffset is -1. P picture QP offsets are alternatively equal to 4, 5, and are set to 1 every 8 pictures.

##### 6.4.8.3.5	SCC-02: Screen Content Profile with no fixed Intra

To generate the anchor bitstreams SCM-8.8 is used.

The settings are defined in the attached configuration file scc-02.cfg.

In summary, each source sequence is encoded with

-	QP: [22, 27, 32, 37, 42]

-	IntraPeriod such that 1 second is achieved

- 	DecodingRefreshType: (2) IDR

-  IntraQPOffset and P pictures QPoffsets are set equal to 0

-  Each P picture refers to immediately preceding pictures in display order.

### 6.4.9	Anchor Results

HEVC anchor streams are provided according to the key system here:

- https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Anchors/Scenario-3

HEVC anchor results are provided with the appropriate keys as defined in Table 6.4.8.3.1-1

- in the attached csv files

- https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Anchors/Metrics/Scenario-3/

Editor’s Note:

- Please cross-check of the results latest until next SA4#113e meeting

### 6.4.10	Additional Information and Performance Data

In JCTVC-AA1006 [40], verification test report for HEVC screen content coding extensions, “the coding performance of HEVC screen content model (SCM) reference software with the SCC extensions enabled is compared with that of HEVC test model (HM) without the SCC extensions, as well as with the AVC joint model (JM) reference software in both lossy and mathematically lossless compression modes using All-Intra (AI), Random Access (RA), and Low-delay B (LB) encoding structures and using similar encoding rate-distortion optimization techniques.

BD-rate savings of SCM over JM and SCM-w/o-SCC for RGB, YUV 4:4:4, and YUV 4:2:0 colour sampling formats were calculated by using the actual 648 test points that were used in the subjective testing across 6 JCT-VC sequences.

The document contains details on sequences, test parameters and configuration files, as well as subjective results. In the context of this study, only the objective results for lossy Low-delay B are reported in Table 6.4.10-1.

Table 6.4.10-1: BD-rate savings of SCM over JM and HM for LB coding configuration

## 6.5	Scenario 4: Messaging and Social Sharing

### 6.5.1	Motivation

According to the 2020 Mobile Internet Phenomena Report from Sandvine [9] Video traffic continues to grow worldwide, and the increasing popularity of mobile consumers sharing video has not only caused growth in downstream traffic, but also in upstream traffic as well. Instagram ™ grew in the upstream as more consumers share images and videos. TikTok ™, Snapchat ™ (video), FaceTime ™, and even Facebook ™ Live were all in the top 50 applications worldwide on the upstream that are video-sharing-centric. Messaging applications, especially on the upstream, continue to become a critical part of the mobile experience, replacing old style text messaging, and increasingly are video-based. Four of the top 20 applications on the upstream are messaging apps.

Some typical examples and restrictions in April 2020 are provided in the following:

1.	WhatApp™ [25]

a.	The maximum size of the video that you can share is 16 MB.

b.	Various container formats that are supported by include MP4, MKV, AVI, 3GP, and MOV. H.264/AVC video codec and AAC audio codec are needed today.

2.	YouTube™ [26]

a.	H.264/AVC is the recommended codec with the following settings

i.	Progressive scan

ii.	High Profile

iii.	2 consecutive B frames

iv.	Closed GOP. GOP of half the frame rate.

v.	CABAC

vi.	Variable bitrate. No bitrate limit required, though we offer recommended bit rates below for reference

vii.	Chroma subsampling: 4:2:0

b.	Resolution Formats: 360p, 480p, 720p, 1080p, 1440p, 2160p

c.	Both SDR and HDR are possible

d.	The standard aspect ratio is 16:9

3.	Facebook Live ™ [27]. To live stream on Facebook™, these video format guidelines are provided:

a.	Recommended max bit rate is 4000 Kbps (4 mbps).

b.	Max: 1080p (1920x1080) resolution, at 60 frames per second.

c.	An I-frame (keyframe) must be sent at least every 2 seconds throughout the stream.

d.	H264 encoded video.

4.	TikTok™ [28], some video restrictions

a.	Upload from Android, up to 72 MB at most. upload videos from iOS up to 287.6 MB.

b.	Video dimensions should be 1080 x 1920

c.	MP4 or MOV file format. Video should be H.264/AVC encoded

5.	Snapchat™ [25][2)], The latest information from 2018

a.	Recommended size: 1080 by 1920 pixels (9:16 aspect ratio)

b.	Recommended specs: .MP4 or MOV, H.264 encoded, maximum file size 1GB

According to Sandvine's report [9], sharing and uploading content as part of social sharing is predominantly pictures and videos that uploaded directly into a cloud and uploaded to one or many social networks, and then discussed (or shared again) over messaging networks. The relevant quality-of-experience factors include the quality of shared content, the time it takes to upload, the costs associated with the upload and also the processing and battery consumption requirements on the device to prepare the content for upload.

In another activity, three large operators released a 5G messaging white paper [31] to promote enhanced messaging services based on Rich Communication Services (RCS). This also shows the relevance of operator-based messaging services. Generally, uplink resources are even more precious and costly in 5G network operation and hence efficient technologies are vital for mass-scale services. This aspect is also considered by GSMA RCS Universal Profile specification, promoted as the industry standard for RCS Business Messaging, ensuring the telecoms industry remains at the centre of digital communications [34].

### 6.5.2	Description of the Anticipated Application

In the context of 3GPP services, 5G Media Streaming [13] provides the following encoding benchmark capabilities:

-	HEVC-FullHD-Enc: the capability to encode a video signal with

-	up to 133,693,440 luma samples per second, and

-	up to a luma picture size of 2,228,224 samples, and

-	up to 240 frames per second, and

-	the Chroma format being 4:2:0, and

-	the bit depth being either 8 or 10 bit,

to a bitstream that is decodable by a decoder that is HEVC-FullHD-Dec capable as defined in clause 4.2.2.1 of TS26.511 and defined as the capability to decode H.265 (HEVC) Main10 Profile, Main Tier, Level 4.1 [3] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, and general_frame_only_constraint_flag equal to 1.

Based on the considerations in clause 6.5.1, it is also recommended to take into account the AVC-FullHD-Enc capabilities as defined in TS26.511 [13]:

-	AVC-FullHD-Enc: the capability to encode a video signal with

-	up to 245,760 macroblocks per second, and

-	up to a frame size of 8,192 macroblocks, and

-	up to 240 frames per second, and

-	the Chroma format being 4:2:0, and

-	the bit depth being 8 bit,

to a bitstream that is decodable by a decoder that is AVC-HD-Dec capable as defined in clause 4.2.1.1 of TS26.511 and defined as the capability to decode H.264 (AVC) Progressive High Profile Level 4.0 [2] bitstreams.

Based on future expectations of higher quality uploads, it is also recommened to take into account the HEVC-UHD-Enc capabilities as defined in TS26.511 [13]:

-	HEVC-UHD-Enc: the capability to encode a video signal with

-	up to 534,773,760 luma samples per second, and

-	up to a luma picture size of 8,912,896 samples, and

-	up to 480 frames per second, and

-	the Chroma format being 4:2:0, and

-	the bit depth being either 8 or 10 bit,

to a bitstream that is decodable by a decoder that is HEVC-UHD-Dec capable as defined in clause 4.2.2.1 of TS26.511 and defined as the capability the capability to decode H.265 (HEVC) Main10 Profile, Main Tier, Level 5.1[3] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, and general_frame_only_constraint_flag equal to 1.

The considered scenario is the uploading and uplink streaming into the ISO/BMFF and CMAF container formats. Important aspects that are expected to be considered when evaluating a codec in the context of this:

-	Quality and Coding Efficiency:

-	The ability to compress a video sequence targeting the maximum file size and maintaining high quality.

-	The ability to compress a video stream in real time to the available uplink streaming resources.

- 	Considered settings for encoding:

-	Regular random access at least every 2 seconds, preferably more often

-	No specific encoding latency constraints are applicable

-	Encoding in this scenario is typically done as

-	Real-time encoding for social sharing

-	Offline encoding for messaging

- 	UE-based Encoding

### 6.5.3	Source Format Properties

Table 6.5.3-1 provides an overview of the different source signal properties for Social Sharing and Messaging. This information is used to select proper test sequences.

Table 6.5.3-1 Source Format Properties for Social sharing scenario

| Source format properties | Social Sharing |
| --- | --- |
| Spatial resolution |  | 1920 x 1080 (Permitted encoding formats: 1920 × 1080, 1280 × 720, 854 × 480) |  |
| Chroma format |  | Y’CbCr |  |
| Chroma subsampling |  | 4:2:0 |  |
| Picture aspect ratio |  | 16:9 |  |
| Frame rates |  | 25, 30, 50, 60 Hz |  |
| Bit depth |  | 8, 10 |  |
| Colour space formats |  | BT.709, BT.2020 |  |
| Transfer characteristics |  | BT.709, BT.2100 (HDR) |  |

### 6.5.4	Encoding and Decoding Constraints

Table 6.5.4-1 provides an overview of encoding and decoding constraints for H.264/AVC Full HD and H.265/HEVC for Social Sharing and Messaging scenario. This information supports the definition of detailed anchor conditions.

Table 6.5.4-1 Encoding and Decoding Configurations

| Encoding and Decoding Constraints | H.264/AVC Full HD | H.265/HEVC Full HD |
| --- | --- | --- |
| Relevant Codec and Codec Profile/Levels | H.264/AVC Progressive High Profile Level 4.0 [7] | H.265/HEVC Main-10 Profile Level 4.0 [8] |
| Random access frequency | 1 and 5 seconds | 1 and 5 seconds |
| Bit rates and quality configuration | B = {0.5, 1, 2, 5} Mbps Capped-VBR (social sharing) and VBR (messaging) Fixed QP | B = {0.5, 1, 2, 5} Mbps Capped-VBR (social sharing) and VBR (messaging) Fixed QP |
| Bit rate parameters (CBR, VBR, CAE, HRD parameters) | No latency requirements beyond RAP so picture reordering allowed | No latency requirements beyond RAP so picture reordering allowed |
| Latency requirements and specific encoding settings | real-time encoding (social sharing), offline encoding (messaging) | real-time encoding (social sharing), offline encoding (messaging). |
| Encoding complexity context | H.264/AVC Main Profile Level 4.0 [7] | H.265/HEVC Main-10 Profile Level 4.0 [8] |
| Required decoding capabilities | H.264/AVC Main Profile Level 4.0 [7] | H.265/HEVC Main-10 Profile Level 4.0 [8] |

### 6.5.5	Performance Metrics

All metrics as defined in clause 5.5 apply.

### 6.5.6	Interoperability Considerations

tbd

### 6.5.7	Reference Sequences

Table 6.5.7-1 provides the selected reference sequences for this scenario. Keys are identified to refer to the sequences in the context of the scenario. The sequences are named and a reference to the details of the sequence is provided. A justification is provided, why this sequence is selected.

Editor’s Note: This is not yet complete.

Table 6.5.7-1 Reference Sequences for Social sharing and messaging

| Key | Name | Reference | Justification/Comment |
| --- | --- | --- | --- |
| S4-R1 |  | Annex C.X |  |
| S4-R2 |  | Annex C.X |  |
| S4-R3 |  | Annex C.X |  |
| S4-R4 |  | Annex C.X |  |
| S4-R5 |  | Annex C.X |  |
| S4-R6 |  | Annex C.X |  |

### 6.5.8	Anchor Definition

#### 6.5.8.1	Overview

This clause provides details on how to generate the anchors for the Social sharing and messaging scenario.

#### 6.5.8.2	H.264/AVC Anchors

##### 6.5.8.2.1	Overview

Table 6.5.8.2.1-1 provides an overview of the H.264/AVC anchor tuples. Keys are identified to refer to the anchors in the context of the scenario.

Editor’s Note: This is not yet complete.

Table 6.5.8.2.1-1 Anchor Tuple generation with H.264/AVC for Social sharing and messaging

| Key | Clause | Reference Sequence | Reference Encoder | Configuration | Variations | Anchor Key |
| --- | --- | --- | --- | --- | --- | --- |
| S4-A1-264 | 6.5.8.2.2 | S4-R1 | JM19.0 | JM-0X | QP = [17, 20, 23, 26, 29, 32] | S4-A1-264-<QP> |
| S4-A2-264 | 6.5.8.2.2 | S4-R2 | JM19.0 | JM-0Y | QP = [17, 20, 23, 26, 29, 32] | S4-A2-264-<QP> |
| S4-A3-264 | 6.5.8.2.2 | S4-R3 | JM19.0 | JM-0Z | QP = [17, 20, 23, 26, 29, 32] | S4-A2-264-<QP> |

##### 6.5.8.2.2	JM-0X

This anchor tuple produces an anchor over 60 seconds with a typical configuration of H.264/AVC:

-	Profile H.264/AVC Progressive High-Profile Level 4.0 [7]

-	Random access and switching at 1 second interval

-	Fixed QP settings

-	IPPPP GOP structures

#### 6.5.8.3	H.265/HEVC Anchors

##### 6.5.8.3.1	Overview

Table 6.5.8.3.1-1 provides an overview of the H.265/HEVC anchor tuples. Keys are identified to refer to the anchors in the context of the scenario.

Editor’s Note: This is not yet complete.

Table 6.5.8.3.1-1 Anchor Tuple generation with H.265/HEVC for Social sharing and messaging

| Key | Clause | Reference Sequence | Reference Encoder | Configuration | Variations | Anchor Key |
| --- | --- | --- | --- | --- | --- | --- |
| S4-A1-265 | 6.5.8.3.2 | S4-R1 | HM16.22 | HM-0X | QP = [22, 25, 28, 31, 34, 37] | S4-A1-265-<QP> |

##### 6.5.8.3.2	HM-0X

tbd

### 6.5.9	Anchor Results

tbd

### 6.5.10	Additional Information and Performance Data

Tbd

## 6.6	Scenario 5: Online Gaming

### 6.6.1	Motivation

According to the 2020 Mobile Internet Phenomena Report from Sandvine [9] gaming is continuing to grow on mobile network. The improved performance of 4G and the coming promise of 5G will continue to drive at least casual gamers to mobile networks.

For games rendered on 2D displays such as smart phones, typical characteristics of rasterized frames produced by the game engine are:

-	Resolution of 720p, 1080p or 4K

-	Framerate of 30fps, 60fps or 120 fps

Typical color bit depth of 8bits (RGB frames) but higher bit depth may be offered for HDR compatible games.

Rasterized frames are directly passed to a video encoder (typically H.264 but H.265 may be used in a few environments) and content is live encoded to fit target quality. As an example, the following quality categorization may be done:

-	High Quality: 4k at 60/120fps with an average throughput of 60/100 Mbps

-	Main Quality: 1080p at 60/120fps with an average throughput of 30/40 Mbps

-	Low Quality: 720p/1080p at 30fps with an average throughput of 10/12 Mbps

Online gaming for HMD-based consumption was discussed and introduced in detail in TR 26.928 [6]. At least the following use cases are in context of Online gaming:

-	Use Case 5: Untethered Immersive Online Gaming

-	Use Case 6: Immersive Game Spectator Mode

For raster-based split rendering, according to TR 26.928, clause 4.4, rasterized 3D scenes available in frame buffers are provided by the XR engine and need to be encoded, distributed, and decoded. According to clause 4.2.1, relevant formats for frame buffers are 2k by 2k per eye, potentially even higher. Frame rates are expected to be at least 60fps, potentially higher up to 90 fps. The formats of frame buffers are regular texture video signals that are then directly rendered. As the processing is graphics centric, formats beyond commonly used 4:2:0 signals and YUV signals may be considered. It is known from experiments that with H.264/AVC the bitrates are in the order of 50 Mbps per eye buffer. It is expected that this can be reduced to lower bitrates with improved compression tools as for example available for H.265/HEVC. For use case 5 from above and split rendering, encoding is required to be done in low-latency based on the considerations in TR 26.928. For the spectator mode, higher latency may be acceptable.

As an example, a comprehensive set of APIs including high-performance tools, samples and documentation for hardware accelerated video encode and decode on Windows and Linux for NVIDIA™ Video Codec SDK is available [N]. As another example, in a game recording and streaming scenario like streaming to Twitch.tv using Open Broadcaster Software (OBS), encoding being completely offloaded to NVENC makes the graphics engine bandwidth fully available for game rendering. As of May 2020, the following formats are supported for hardware-based encoding as documented on the high-end Turing encoding:

-	H.264 (AVCHD) YUV 4:2:0, YUV 4:4:4, and Lossless, all 8 bit, Max Resolution 4096 x 4096;

-	H.265 (HEVC) YUV 4:2:0, YUV 4:4:4, and Lossless, all 10 bit, Max Resolution 8192 x 8192;

In typical cloud gaming environments, the game server produces rasterized frames at a fixed resolution, framerate and colour bit depth which are negotiated with the player client. Negotiation takes into account game capabilities, player choices and eventually bandwidth constraints.

### 6.6.2	Description of the Anticipated Application

3GPP until now has very restricted set of services but based on the considerations in clause 6.6.1, the following encoding benchmark capabilities are considered for decoding 2D displays:

-	H.264 (AVCHD) YUV 4:2:0, YUV 4:4:4, 8 bit, Max Resolution 1920x1080 and 4096 x 2048

-	H.265 (HEVC) YUV 4:2:0, YUV 4:4:4, 10 bit, Max Resolutions 4096 x 2048, 8192 x 4096

The considered online gaming scenario is low-latency streaming, typically using UDP/IP based distribution to minimize protocol latencies. Important aspects that are expected to be considered when evaluating a codec in the context of this:

-	Quality and Coding Efficiency:

-	The ability to compress traditional computer-generated content.

-	The ability to compress photorealistic computer-generated content.

-	The ability to compress YUV 4:2:0 and 4:4:4 content

- 	Considered settings for encoding:

-	Ultra low latency and Low-latency settings

-	No specific error resilience mechanisms

-	Encoding in this scenario is typically done as

-	Real-time encoding

- 	Cloud-based encoding

### 6.6.3	Source Format Properties

#### 6.6.3.1	Introduction

Video games have different characteristics that are important to take into account when encoding the rasterized frames produced by the game engine. In TR 26.928 [6], clause 4.2.2, a few different types of games and their interaction delay tolerance are documented. However, TR 26.928 [6] does not differentiate the characteristics of the content. This aspect is addressed in the following.

In particular, the following characteristics are important:

-	Dynamicity of content: how frequent rasterized frames change when compared to previous frame

-	Complexity of content: how much content changes between frames and how complex such changes are

-	Type of content: traditional CGI, photo-realistic CGI or natural images/video

Depending on these characteristics as well as the interaction delay tolerance, video games can be organized into different categories as document in the remainder of this clause.

#### 6.6.3.2	Category A: Low/medium dynamicity with low/medium complexity.

This category includes games such as board games, turn-by-turn strategy games, management/simulation games or non-realtime role-playing games (RPG) in which content may not change over several consecutive frames and changes are typically limited.

This category also includes games such as adventure games, casual games, or platform games in which although content may change at every single frame, changes are limited to animation of sprites or simple global movements of the content.

The common characteristics of the games in this category is that their playability can support longer interaction delay tolerance (500 – 1000ms according to TR 26.928 [6]) and their content is typically considered to video encode.

#### 6.6.3.3	Category B: games with high dynamicity and low/medium complexity.

This category includes games such as fighting games, racing games, real-time strategy (RTS) games or real-time RPGs in which content is very dynamic but changes are either limited or simple transforms.

The common characteristics of the games in this category is that their playability requires shorter interaction delay tolerances (100ms according to TR 26.928 [6]) while their content is still considered simple to video encode (with high benefits from prediction coding).

#### 6.6.3.4	Category C: games with high dynamicity and high complexity.

This category includes games such as first-person shooters (FPS), Massive Multiplayer Online (MMO) games and racing games in which content is very dynamic with possibly very significant changes regularly in the content.

The common characteristics of the games in this category is that their playability requires shorter interaction delay tolerances (100ms according to TR 26.928) and their content is typically considered as complex content to video encode.

#### 6.6.3.5	Category D: photo-realistic games or games based on natural images/video.

The main characteristics of the games in this category is that their content is typically considered as more complex content to video encode.

#### 6.6.3.6	Category E: XR game content

For a detailed analysis, please refer to clause 6.6.1 and TR26.928, clause 4.4.

#### 6.6.3.7	Summary

Table 6.6.3.7-1 provides an overview of the different source signal properties for Online Gaming. This information is used to select proper test sequences.

Table 6.6.3.7-1 Online Gaming source properties

| Source format properties | Category A low/med dynamicity & low/med complexity games | Category B high dynamicity & low/med complexity games | Category C high dynamicity & high complexity games | Category D photo-realistic or natural video games | Category E XR games |
| --- | --- | --- | --- | --- | --- |
| Spatial resolution | 1280x720, 1920x1080, 3840x2160 | 1280x720, 1920x1080, 3840x2160 | 1280x720, 1920x1080, 3840x2160 | 1280x720, 1920x1080, 3840x2160 | 1920x1080, 2048x1024, 3840x2160, 4096x2048, 8192x4096 |
| Chroma format | Y’CbCr | Y’CbCr | Y’CbCr | Y’CbCr | Y’CbCr |
| Chroma subsampling | 4:2:0, 4:4:4 | 4:2:0, 4:4:4 | 4:2:0, 4:4:4 | 4:2:0, 4:4:4 | 4:2:0, 4:4:4 |
| Picture aspect ratio | 16:9 | 16:9 | 16:9 | 16:9 | 16:9; 2:1 |
| Frame Buffers | 1 | 1 | 1 | 1 | 2 (1 per eye) |
| Frame rates | 30, 50, 60 Hz | 30, 50, 60, 90, 120 Hz | 30, 50, 60, 90, 120 Hz | 30, 50, 60, 90, 120 Hz | 30, 50, 60, 90, 120 Hz |
| Bit depth | 8 | 8 | 8, 10 | 8, 10 | 8, 10 |
| Colour space formats | BT.709, BT.2020 | BT.709, BT.2020 | BT.709, BT.2020 | BT.709, BT.2020 | BT.709, BT.2020 |
| Transfer characteristics | N/A | N/A | BT.709 (SDR) BT.2100 (HDR) | BT.709 (SDR) BT.2100 (HDR) | BT.709 (SDR) BT.2100 (HDR) |

### 6.6.4	Encoding and Decoding Constraints

Table 6.6.4-1 provides an overview of the different coding tools per profile that may be suitable for coding different types of gaming sequences.

Table 6.6.4-1 Gaming Codec Tools per Profile

| Profile | AVC | HEVC |
| --- | --- | --- |
| main profile | NA | TBD |
| range extension profile | NA | TBD |
| screen content profile | NA | Palette, TBD |

Note: Palette being heavily used in the design of games, this tool should be particularly tested for game content.

For photo-realistic games (Category D), certain "screen content" tools may not provide much benefits for this type of game content and would need to be tested.

Table 6.6.4-2 provides an overview of encoding and decoding constraints for H.264/AVC Full HD and H.265/HEVC for online gaming. This information supports the definition of detailed anchor conditions.

Table 6.6.4-2 Encoding and Decoding Configurations for Online Gaming

| Encoding and Decoding Constraints | AVC | HEVC |
| --- | --- | --- |
| Relevant Codec and Codec Profile/Levels according to TS26.116 and TS26.511. | H.264/AVC Progressive High Profile Level 4.0 [7] | H.265/HEVC Main-10 Profile Level 4.1, Level 5.1, Level 6.1 |
| RAP period | 1 second, infinite | 1 second, infinite |
| Bit rate parameters (CBR, VBR, CAE, HRD parameters) | Fixed QP | Fixed QP |
| Latency requirements and specific encoding settings | Low-latency requirements Low-delay configuration With and without use of future reference frames | Low-latency requirements, Low-delay-P Low-delay-B With (moderate latency to non-critical latency games) and without (ultra low and low latency game) use of future reference frames. |
| Encoding complexity context | real-time encoding | real-time encoding. |
| Required decoding capabilities | H.264/AVC Main Profile Level 4.0 [7] | H.265/HEVC Main-10 Profile Level 4.1, 5.1, 6.1 [8] |
| NOTE: Depending on the type of game (First person, Massive Multi-Player Online Role Play Game, Strategy) the latency requirements differs, usually ranging from less than 50ms to more than 200ms, thus leading to different encoding constraints and configuration. |  |  |  |  |

[Table 6.6.5-2 provides an overview of latency constraints for each game category and identifies which coding tools are applicable and potentially useful for each category.

Table 6.6.5-2 Latency constraints tools for different game categories

| Latency Constraints | Category A | Category B | Category C | Category D | Category E |
| --- | --- | --- | --- | --- | --- |
| Low latency requirement | optional | Yes | Yes | Yes/No | Yes |
| Low delay P | optional | required | required | optional | required |
| Low delay B | optional | required | required | optional | required |
| Use of future reference frames | optional | No | No | optional | No |

]

### 6.6.5	Performance Metrics

All metrics as defined in clause 5.5 apply.

### 6.6.6	Interoperability Considerations

tbd

### 6.6.7	Reference Sequences

Reference sequences illustrating the Online Gaming scenario should ideally contain synthetic content with detailed textures and realistic movements. This should include FPS, RPG and Strategy games. For candidate test sequences, please refer to Annex C.2.

Table 6.6.7-1 provides the selected reference sequences for this scenario. Keys are identified to refer to the sequences in the context of the scenario. The sequences are named and a reference to the details of the sequence is provided. A justification is provided, why this sequence is selected.

The details are also provided here: http://dash.akamaized.net/WAVE/3GPP/5GVideo/Anchors/Scenario-5/reference-sequence.csv.

Table 6.6.7-1 Reference Sequences for Online Gaming

| Key | Name | Reference | Justification/Comment |
| --- | --- | --- | --- |
| S5-R01 | AOV | Annex C.2.2 | FullHD Sequence at 60fps |
| S5-R02 | Baolei-Man | Annex C.2.3 | FullHD Sequence at 60fps |
| S5-R03 | Baolei-Woman | Annex C.2.4 | FullHD Sequence at 60fps |
| S5-R04 | Baolei-Balloon | Annex C.2.5 | 4K Sequence at 60fps |
| S5-R05 | Baolei-Yard | Annex C.2.6 | 4K Sequence at 60fps |
| S5-R06 | Jianling-Temple | Annex C.2.7 | FullHD Sequence at 60fps |
| S5-R07 | Jianling-Beach | Annex C.2.8 | FullHD Sequence at 60fps |
| S5-R08 | Heroes-of-the-Storm | Annex C.2.9 | FullHD Sequence at 30fps |
| S5-R09 | Project-CARS | Annex C.2.10 | FullHD Sequence at 30fps |
| S5-R10 | World-of-WarCraft | Annex C.2.11 | FullHD Sequence at 30fps |
| S5-R11 | MineCraft | Annex C.2.12 | FullHD Sequence at 60fps |
| S5-R12 | CS-GO | Annex C.2.13 | FullHD Sequence at 60fps |
| S5-R13 | StarCraft | Annex C.2.14 | FullHD Sequence at 60fps |

### 6.6.8	Anchor Definition

#### 6.6.8.1	Overview

This clause provides details on how to generate the anchors for the Online gaming scenario.

#### 6.6.8.2	H.264/AVC Anchors

##### 6.6.8.2.1	Overview

Table 6.6.8.2.1-1 provides an overview of the H.264/AVC anchor tuples. Keys are identified to refer to the anchors in the context of the scenario.

Editor’s Note: this still needs updates

Table 6.6.8.2.1-1 Anchor Tuple generation with H.264/AVC for Online Gaming Scenario

| Key | Clause | Reference Sequence | Reference Encoder | Configuration | Variations | Anchor Key |
| --- | --- | --- | --- | --- | --- | --- |
| S5-A1-264 | 6.6.8.2.2 | S5-R1 | JM19.0 | JM-01 | QP = [17, 20, 23, 26, 29, 32] | S5-A1-264-<QP> |
| S5-A2-264 | 6.6.8.2.2 | S5-R2 | JM19.0 | JM-02 | QP = [17, 20, 23, 26, 29, 32] | S5-A2-264-<QP> |
| S5-A3-264 | 6.6.8.2.2 | S5-R3 | JM19.0 | JM-03 | QP = [17, 20, 23, 26, 29, 32] | S5-A2-264-<QP> |

##### 6.6.8.2.2	tbd

tbd

#### 6.6.8.3	H.265/HEVC Anchors

##### 6.6.8.3.1	Overview

Table 6.6.8.3.1-1 provides an overview of the H.265/HEVC anchor tuples. Keys are identified to refer to the anchors in the context of the scenario.

The details are also provided here: http://dash.akamaized.net/WAVE/3GPP/5GVideo/Anchors/Scenario-5/anchors.csv.

Table 6.5.8.3.1-1 Anchor Tuple generation with H.265/HEVC for Online Gaming Scenario

| Key | Clause | Reference Sequence | Reference Encoder | Configuration | Variations | Anchor Key |
| --- | --- | --- | --- | --- | --- | --- |
| S5-A01-265 | 6.6.8.3.2 | S5-R01 | HM16.22 | HM-01 | [22, 27, 32, 37] | S5-A01-265-<QP> |
| S5-A02-265 | 6.6.8.3.2 | S5-R02 | HM16.22 | HM-01 | [22, 27, 32, 37] | S5-A02-265-<QP> |
| S5-A03-265 | 6.6.8.3.2 | S5-R03 | HM16.22 | HM-01 | [22, 27, 32, 37] | S5-A03-265-<QP> |
| S5-A04-265 | 6.6.8.3.2 | S5-R04 | HM16.22 | HM-01 | [22, 27, 32, 37] | S5-A04-265-<QP> |
| S5-A05-265 | 6.6.8.3.2 | S5-R05 | HM16.22 | HM-01 | [22, 27, 32, 37] | S5-A05-265-<QP> |
| S5-A06-265 | 6.6.8.3.2 | S5-R06 | HM16.22 | HM-01 | [22, 27, 32, 37] | S5-A06-265-<QP> |
| S5-A07-265 | 6.6.8.3.2 | S5-R07 | HM16.22 | HM-01 | [22, 27, 32, 37] | S5-A07-265-<QP> |
| S5-A08-265 | 6.6.8.3.2 | S5-R08 | HM16.22 | HM-01 | [22, 27, 32, 37] | S5-A08-265-<QP> |
| S5-A09-265 | 6.6.8.3.2 | S5-R09 | HM16.22 | HM-01 | [22, 27, 32, 37] | S5-A09-265-<QP> |
| S5-A10-265 | 6.6.8.3.2 | S5-R10 | HM16.22 | HM-01 | [22, 27, 32, 37] | S5-A10-265-<QP> |
| S5-A11-265 | 6.6.8.3.2 | S5-R11 | HM16.22 | HM-01 | [22, 27, 32, 37] | S5-A11-265-<QP> |
| S5-A12-265 | 6.6.8.3.2 | S5-R12 | HM16.22 | HM-01 | [22, 27, 32, 37] | S5-A12-265-<QP> |
| S5-A13-265 | 6.6.8.3.2 | S5-R13 | HM16.22 | HM-01 | [22, 27, 32, 37] | S5-A13-265-<QP> |
| S5-A14-265 | 6.6.8.3.3 | S5-R01 | HM16.22 | HM-02 | [22, 27, 32, 37] | S5-A14-265-<QP> |
| S5-A15-265 | 6.6.8.3.3 | S5-R02 | HM16.22 | HM-02 | [22, 27, 32, 37] | S5-A15-265-<QP> |
| S5-A16-265 | 6.6.8.3.3 | S5-R03 | HM16.22 | HM-02 | [22, 27, 32, 37] | S5-A16-265-<QP> |
| S5-A17-265 | 6.6.8.3.3 | S5-R04 | HM16.22 | HM-02 | [22, 27, 32, 37] | S5-A17-265-<QP> |
| S5-A18-265 | 6.6.8.3.3 | S5-R05 | HM16.22 | HM-02 | [22, 27, 32, 37] | S5-A18-265-<QP> |
| S5-A19-265 | 6.6.8.3.3 | S5-R06 | HM16.22 | HM-02 | [22, 27, 32, 37] | S5-A19-265-<QP> |
| S5-A20-265 | 6.6.8.3.3 | S5-R07 | HM16.22 | HM-02 | [22, 27, 32, 37] | S5-A20-265-<QP> |
| S5-A21-265 | 6.6.8.3.3 | S5-R08 | HM16.22 | HM-02 | [22, 27, 32, 37] | S5-A21-265-<QP> |
| S5-A22-265 | 6.6.8.3.3 | S5-R09 | HM16.22 | HM-02 | [22, 27, 32, 37] | S5-A22-265-<QP> |
| S5-A23-265 | 6.6.8.3.3 | S5-R10 | HM16.22 | HM-02 | [22, 27, 32, 37] | S5-A23-265-<QP> |
| S5-A24-265 | 6.6.8.3.3 | S5-R11 | HM16.22 | HM-02 | [22, 27, 32, 37] | S5-A24-265-<QP> |
| S5-A25-265 | 6.6.8.3.3 | S5-R12 | HM16.22 | HM-02 | [22, 27, 32, 37] | S5-A25-265-<QP> |
| S5-A26-265 | 6.6.8.3.3 | S5-R13 | HM16.22 | HM-02 | [22, 27, 32, 37] | S5-A26-265-<QP> |
| S5-A27-265 | 6.6.8.3.4 | S5-R01 | SCC8.8 | SCC-01 | [22, 27, 32, 37] | S5-A27-265-<QP> |
| S5-A28-265 | 6.6.8.3.4 | S5-R02 | SCC8.8 | SCC-01 | [22, 27, 32, 37] | S5-A28-265-<QP> |
| S5-A29-265 | 6.6.8.3.4 | S5-R03 | SCC8.8 | SCC-01 | [22, 27, 32, 37] | S5-A29-265-<QP> |
| S5-A30-265 | 6.6.8.3.4 | S5-R04 | SCC8.8 | SCC-01 | [22, 27, 32, 37] | S5-A30-265-<QP> |
| S5-A31-265 | 6.6.8.3.4 | S5-R05 | SCC8.8 | SCC-01 | [22, 27, 32, 37] | S5-A31-265-<QP> |
| S5-A32-265 | 6.6.8.3.4 | S5-R06 | SCC8.8 | SCC-01 | [22, 27, 32, 37] | S5-A32-265-<QP> |
| S5-A33-265 | 6.6.8.3.4 | S5-R07 | SCC8.8 | SCC-01 | [22, 27, 32, 37] | S5-A33-265-<QP> |
| S5-A34-265 | 6.6.8.3.4 | S5-R08 | SCC8.8 | SCC-01 | [22, 27, 32, 37] | S5-A34-265-<QP> |
| S5-A35-265 | 6.6.8.3.4 | S5-R09 | SCC8.8 | SCC-01 | [22, 27, 32, 37] | S5-A35-265-<QP> |
| S5-A36-265 | 6.6.8.3.4 | S5-R10 | SCC8.8 | SCC-01 | [22, 27, 32, 37] | S5-A36-265-<QP> |
| S5-A37-265 | 6.6.8.3.4 | S5-R11 | SCC8.8 | SCC-01 | [22, 27, 32, 37] | S5-A37-265-<QP> |
| S5-A38-265 | 6.6.8.3.4 | S5-R12 | SCC8.8 | SCC-01 | [22, 27, 32, 37] | S5-A38-265-<QP> |
| S5-A39-265 | 6.6.8.3.4 | S5-R13 | SCC8.8 | SCC-01 | [22, 27, 32, 37] | S5-A39-265-<QP> |
| S5-A40-265 | 6.6.8.3.5 | S5-R01 | SCC8.8 | SCC-02 | [22, 27, 32, 37] | S5-A40-265-<QP> |
| S5-A41-265 | 6.6.8.3.5 | S5-R02 | SCC8.8 | SCC-02 | [22, 27, 32, 37] | S5-A41-265-<QP> |
| S5-A42-265 | 6.6.8.3.5 | S5-R03 | SCC8.8 | SCC-02 | [22, 27, 32, 37] | S5-A42-265-<QP> |
| S5-A43-265 | 6.6.8.3.5 | S5-R04 | SCC8.8 | SCC-02 | [22, 27, 32, 37] | S5-A43-265-<QP> |
| S5-A44-265 | 6.6.8.3.5 | S5-R05 | SCC8.8 | SCC-02 | [22, 27, 32, 37] | S5-A44-265-<QP> |
| S5-A45-265 | 6.6.8.3.5 | S5-R06 | SCC8.8 | SCC-02 | [22, 27, 32, 37] | S5-A45-265-<QP> |
| S5-A46-265 | 6.6.8.3.5 | S5-R07 | SCC8.8 | SCC-02 | [22, 27, 32, 37] | S5-A46-265-<QP> |
| S5-A47-265 | 6.6.8.3.5 | S5-R08 | SCC8.8 | SCC-02 | [22, 27, 32, 37] | S5-A47-265-<QP> |
| S5-A48-265 | 6.6.8.3.5 | S5-R09 | SCC8.8 | SCC-02 | [22, 27, 32, 37] | S5-A48-265-<QP> |
| S5-A49-265 | 6.6.8.3.5 | S5-R10 | SCC8.8 | SCC-02 | [22, 27, 32, 37] | S5-A49-265-<QP> |
| S5-A50-265 | 6.6.8.3.5 | S5-R11 | SCC8.8 | SCC-02 | [22, 27, 32, 37] | S5-A50-265-<QP> |
| S5-A51-265 | 6.6.8.3.5 | S5-R12 | SCC8.8 | SCC-02 | [22, 27, 32, 37] | S5-A51-265-<QP> |
| S5-A52-265 | 6.6.8.3.5 | S5-R13 | SCC8.8 | SCC-02 | [22, 27, 32, 37] | S5-A52-265-<QP> |

##### 6.6.8.3.2	HM-01: Main 10 Profile with no fixed Intra

To generate the anchor bitstreams, HM16.22 is used.

The settings are defined

in the attached configuration file hm-01.cfg.

In summary, each source sequence is encoded with the following changes:

-	QP: [22, 27, 32, 37]

-	IntraPeriod with no fix interval

-  GOPSize is equal to 8. Each B picture refers to up to 4 preceding pictures in display order

-  IntraQPOffset is -1. B picture QP offsets are alternatively equal to 4, 5, and are set to 1 every 8 pictures.

##### 6.6.8.3.3	HM-02: Main 10 Profile with fixed Intra every second

To generate the anchor bitstreams, HM16.22 is used.

The settings are defined in the attached configuration file hm-02.cfg.

In summary, each source sequence is encoded with the following changes:

-	QP: [22, 27, 32, 37]

-	IntraPeriod such that 1 second is achieved

-  DecodingRefreshType: (2) IDR

-  IntraQPOffset and B pictures QPoffsets are set equal to 0

-  Each B picture refers to immediately preceding pictures in display order.

##### 6.6.8.3.4	SCC-01: Screen Content Profile with no fixed Intra

To generate the anchor bitstreams SCM-8.8 is used.

The settings are defined in the attached configuration file scc-01.cfg.

In summary, each source sequence is encoded with

-	QP: [22, 27, 32, 37]

-	IntraPeriod with no fix interval

-  	GOPSize is equal to 8. Each P picture refers to up to 4 preceding pictures in display order

-  	IntraQPOffset is -1. P picture QP offsets are alternatively equal to 4, 5, and are set to 1 every 8 pictures.

##### 6.6.8.3.5	SCC-02: Screen Content Profile with no fixed Intra

To generate the anchor bitstreams SCM-8.8 is used.

The settings are defined in the attached configuration file scc-02.cfg.

In summary, each source sequence is encoded with

-	QP: [22, 27, 32, 37]

-	IntraPeriod such that 1 second is achieved

- 	DecodingRefreshType: (2) IDR

-  IntraQPOffset and P pictures QPoffsets are set equal to 0

-  Each P picture refers to immediately preceding pictures in display order.

### 6.5.9	Anchor Results

HEVC anchor streams are provided according to the key system here:

- https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Anchors/Scenario-5

HEVC anchor results are provided with the appropriate keys as defined in Table 6.4.8.3.1-1

- in the attached csv files

- https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Anchors/Metrics/Scenario-5/

Editor’s Note:

- Please cross-check of the results latest until next SA4#113e meeting

### 6.5.10	Additional Information and Performance Data

Tbd

# 7	Characterization for Existing Codecs

## 7.1	Introduction

This clause provides a characterization of existing codecs. This serves pre-dominantly as an example to introduce the characterization framework.

## 7.2	H.265/HEVC Characterization against H.264/AVC

Do a first characterization, also to show the how the results are preferably presented.

# 8	Initial Information on new Codecs

## 8.1	Introduction

This clause collects initial information on how new codecs under development in ISO/IEC SC29 WG11 (MPEG)/JVET (in particular including VVC and EVC) may meet the above criteria based on the characterization results provided for example by ISO/IEC SC29 WG11 (MPEG)/JVET.

Editor’s Note: It is expected that the preliminary coding performances documented in the following clauses for VCC and EVC will be updated by the latest results from JVET and MPEG when made available.

## 8.2	Versatile Video Coding (VVC)

### 8.2.1	Overview

ITU-T SG 16 informed that the development of the new Versatile Video Coding (VVC) standard has been completed for Consent at the June-July 2020 meeting of ITU-T SG16 and is also thereafter undergoing final approval in ISO/IEC JTC 1/SC 29/WG 11 (MPEG). After final approval, the dual reference to the jointly developed VVC standard will be Rec. ITU-T H.266 | ISO/IEC 23090-3 [38].

VVC is the latest in a series of such jointly developed ITU-T Recommendations and International Standards for video coding, and is the direct successor to HEVC (Rec. ITU-T H.265 | ISO/IEC 23008-2) [8] and AVC (Rec. ITU-T H.264 | ISO/IEC 14496-10) [7]. VVC provides significant coding efficiency improvements over HEVC. Plans are underway to conduct a verification test with formal subjective testing to confirm that VVC achieves about a 50% bit rate reduction vs. HEVC for equal subjective video quality. Test results demonstrate that VVC provides about a 40% bit rate reduction for 4K/UHD test sequences using objective metrics. Application areas especially targeted for the use of VVC include ultra-high definition 4K and 8K video, video with a high dynamic range and wide colour gamut, and video for immersive media applications such as 360° omnidirectional video, as well as conventional standard-definition and high-definition video content. In addition to improving coding efficiency, VVC also provides highly flexible syntax supporting such use cases as subpicture bitstream extraction, bitstream merging, and layered coding scalability.

The VVC standard includes the specification of a "Main 10" profile that supports 8- and 10-bit 4:2:0 video, a "Main 10 4:4:4" profile with 4:4:4 and 4:2:2 format support, corresponding "Multilayer Main 10" and "Multilayer Main 10 4:4:4" profiles with support for layered coding, and "Main 10 Still Picture" and "Main 10 4:4:4 Still Picture" profiles for still image coding employing the same coding tools as in the corresponding video profiles.

ITU-T also announced AAP Consent for Rec. ITU-T H.274 “Versatile supplemental enhancement information for coded video bitstreams” (VSEI), developed jointly as twin text with ISO/IEC JTC 1/SC 29/WG 11 as ISO/IEC 23002-7 [39]. VSEI specifies the syntax and semantics of video usability information (VUI) parameters and supplemental enhancement information (SEI) messages for use with coded video bitstreams. VSEI is particularly intended for use with VVC, although it is drafted in a manner intended to be sufficiently generic that it may also be used with other types of coded video bitstreams.

### 8.2.2	External subjective tests results

The document [61] reports verification test results comparing VVC to its predecessor HEVC on ultra high definition (UHD, a.k.a. 4K, 3840×2160) standard dynamic range (SDR) video content using formal subjective visual quality assessment testing. The purpose of the verification test was to confirm that the coding efficiency objective for the VVC standard has been met achieving a substantial bit-rate reduction for the same level of subjective visual quality relative to the HEVC Main Profile. As anticipated in the test plan, in addition to using the HM reference software encoder for HEVC and the VTM reference software encoder for VVC, which used essentially the same rate-distortion optimization encoding techniques, another VVC encoder that uses alternative techniques for subjective quality optimization and faster encoding had also become available for study and was included in the test – namely the VVenC open-source VVC encode [62]. The VVenC encoder, although still a preliminary implementation produced only two months following the completion of the standard, was used to represent an example of practical encoding as may be found in product implementations and is reported to be more than 100 times faster than the VTM encoder [63]. The compression performance of the HEVC reference software HM-16.22, the VVC reference software VTM-10.0, and the open-source VVC implementation VVenC-0.1.0, were compared for UHD SDR content using a random-access (RA) configuration suitable for streaming or broadcast applications.

The testing used the degradation category rating (DCR) test method (as in ITU-T P.910) [64] with an 11-point impairment scale (as in Rec. ITU-R BT.500). The results of a visual assessment of VVC compared to HEVC by naïve test subjects are reported. The assessment included five test sequences encoded in a random-access configuration with a random-access interval of 1.07 seconds. The measured mean opinion score (MOS) figures indicate a significant improvement of VVC relative to HEVC for both VVC implementations, VTM-10.0 and VVenC-0.1.0, resulting in overall average bit-rate savings estimates of 43% and 49%, respectively.

The SDR UHD subjective test was carried out at the following test sites:

- GBTech, Rome, IT

- RWTH Aachen University, Aachen, DE

The arrangements for the two test sites are shown in Table 8.2.2-1.

Table 8.2.2-1 Arrangements for VVC subjective test sites

| Test Site | GBTech | RWTH Aachen University |
| --- | --- | --- |
| Display, size (resolution setting) | LG 65” CX6LA (3840x2160) | Sony PVM X550, 55” (3840x2160) |
| Viewing distance | 2 viewers at 1,5H | 1 viewer at 1.5H |
| Viewing angle | 60° (30° from screen center) | 90° (at screen center) |
| Total number of viewers | 16 (7 females, 9 males; age 18-24), all screened for visual acuity and normal colour vision. | 24 (5 females, 19 males, age 16-34), all screened for visual acuity and normal colour vision. |

Verification tests for SDR HD in random access and low delay, HDR UHD random access and video 360 are still in progress in JVET.

### 8.2.3	Test Configurations and Results for VTM

#### 8.2.3.1	Introduction

For H.266/VVC generated test bitstreams, the following test software has been used:

- VTM11.0: The reference software for H.266/MPEG-I VVC is called VTM (VVC Test Model). The VTM software is maintained and can be downloaded from the repository:  https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM. The release used for tests in the report is version 11.0 available here: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/releases/VTM-11.0

#### 8.2.3.2	Scenario 1: Full HD Streaming

#### 8.2.3.3	Scenario 2: 4K-TV

#### 8.2.3.4	Scenario 3: Screen Content Scenario

##### 8.2.3.4.1	Overview

Table 8.2.3.4.1-1 provides an overview of the H.266/VVC test tuples. Keys are identified to refer to the tests in the context of the scenario.

The details are also provided here: http://dash.akamaized.net/WAVE/3GPP/5GVideo/Tests/VTM/Scenario-3/tests.csv.

Table 8.2.3.4.1-1 Test Tuple generation with H.266/VVC for Screen Content Scenario

| Key | Clause | Reference Sequence | Reference Encoder | Configuration | Variations | Anchor Key |
| --- | --- | --- | --- | --- | --- | --- |
| S3-T01-VTM | 8.2.3.4.2.1 | S3-R01 | VTM11.0 | VTM-01 | QP=[22, 27, 32, 37, 42] | S3-T01-VTM-<QP> |
| S3-T02-VTM | 8.2.3.4.2.1 | S3-R02 | VTM11.0 | VTM-01 | QP=[22, 27, 32, 37, 42] | S3-T02-VTM-<QP> |
| S3-T03-VTM | 8.2.3.4.2.1 | S3-R03 | VTM11.0 | VTM-01 | QP=[22, 27, 32, 37, 42] | S3-T03-VTM-<QP> |
| S3-T04-VTM | 8.2.3.4.2.1 | S3-R04 | VTM11.0 | VTM-01 | QP=[22, 27, 32, 37, 42] | S3-T04-VTM-<QP> |
| S3-T05-VTM | 8.2.3.4.2.1 | S3-R05 | VTM11.0 | VTM-01 | QP=[22, 27, 32, 37, 42] | S3-T05-VTM-<QP> |
| S3-T06-VTM | 8.2.3.4.2.1 | S3-R06 | VTM11.0 | VTM-01 | QP=[22, 27, 32, 37, 42] | S3-T06-VTM-<QP> |
| S3-T07-VTM | 8.2.3.4.2.1 | S3-R07 | VTM11.0 | VTM-01 | QP=[22, 27, 32, 37, 42] | S3-T07-VTM-<QP> |
| S3-T08-VTM | 8.2.3.4.2.1 | S3-R08 | VTM11.0 | VTM-01 | QP=[22, 27, 32, 37, 42] | S3-T08-VTM-<QP> |
| S3-T09-VTM | 8.2.3.4.2.1 | S3-R09 | VTM11.0 | VTM-01 | QP=[22, 27, 32, 37, 42] | S3-T09-VTM-<QP> |
| S3-T10-VTM | 8.2.3.4.2.1 | S3-R10 | VTM11.0 | VTM-01 | QP=[22, 27, 32, 37, 42] | S3-T10-VTM-<QP> |
| S3-T11-VTM | 8.2.3.4.2.1 | S3-R11 | VTM11.0 | VTM-01 | QP=[22, 27, 32, 37, 42] | S3-T11-VTM-<QP> |
| S3-T12-VTM | 8.2.3.4.2.1 | S3-R12 | VTM11.0 | VTM-01 | QP=[22, 27, 32, 37, 42] | S3-T12-VTM-<QP> |
| S3-T13-VTM | 8.2.3.4.2.1 | S3-R13 | VTM11.0 | VTM-01 | QP=[22, 27, 32, 37, 42] | S3-T13-VTM-<QP> |
| S3-T14-VTM | 8.2.3.4.2.1 | S3-R14 | VTM11.0 | VTM-01 | QP=[22, 27, 32, 37, 42] | S3-T14-VTM-<QP> |
| S3-T15-VTM | 8.2.3.4.2.1 | S3-R15 | VTM11.0 | VTM-01 | QP=[22, 27, 32, 37, 42] | S3-T15-VTM-<QP> |
| S3-T16-VTM | 8.2.3.4.2.1 | S3-R16 | VTM11.0 | VTM-01 | QP=[22, 27, 32, 37, 42] | S3-T16-VTM-<QP> |
| S3-T17-VTM | 8.2.3.4.2.2 | S3-R01 | VTM11.0 | VTM-02 | QP=[22, 27, 32, 37, 42] | S3-T17-VTM-<QP> |
| S3-T18-VTM | 8.2.3.4.2.2 | S3-R02 | VTM11.0 | VTM-02 | QP=[22, 27, 32, 37, 42] | S3-T18-VTM-<QP> |
| S3-T19-VTM | 8.2.3.4.2.2 | S3-R03 | VTM11.0 | VTM-02 | QP=[22, 27, 32, 37, 42] | S3-T19-VTM-<QP> |
| S3-T20-VTM | 8.2.3.4.2.2 | S3-R04 | VTM11.0 | VTM-02 | QP=[22, 27, 32, 37, 42] | S3-T20-VTM-<QP> |
| S3-T21-VTM | 8.2.3.4.2.2 | S3-R05 | VTM11.0 | VTM-02 | QP=[22, 27, 32, 37, 42] | S3-T21-VTM-<QP> |
| S3-T22-VTM | 8.2.3.4.2.2 | S3-R06 | VTM11.0 | VTM-02 | QP=[22, 27, 32, 37, 42] | S3-T22-VTM-<QP> |
| S3-T23-VTM | 8.2.3.4.2.2 | S3-R07 | VTM11.0 | VTM-02 | QP=[22, 27, 32, 37, 42] | S3-T23-VTM-<QP> |
| S3-T24-VTM | 8.2.3.4.2.2 | S3-R08 | VTM11.0 | VTM-02 | QP=[22, 27, 32, 37, 42] | S3-T24-VTM-<QP> |
| S3-T25-VTM | 8.2.3.4.2.2 | S3-R09 | VTM11.0 | VTM-02 | QP=[22, 27, 32, 37, 42] | S3-T25-VTM-<QP> |
| S3-T26-VTM | 8.2.3.4.2.2 | S3-R10 | VTM11.0 | VTM-02 | QP=[22, 27, 32, 37, 42] | S3-T26-VTM-<QP> |
| S3-T27-VTM | 8.2.3.4.2.2 | S3-R11 | VTM11.0 | VTM-02 | QP=[22, 27, 32, 37, 42] | S3-T27-VTM-<QP> |
| S3-T28-VTM | 8.2.3.4.2.2 | S3-R12 | VTM11.0 | VTM-02 | QP=[22, 27, 32, 37, 42] | S3-T28-VTM-<QP> |
| S3-T29-VTM | 8.2.3.4.2.2 | S3-R13 | VTM11.0 | VTM-02 | QP=[22, 27, 32, 37, 42] | S3-T29-VTM-<QP> |
| S3-T30-VTM | 8.2.3.4.2.2 | S3-R14 | VTM11.0 | VTM-02 | QP=[22, 27, 32, 37, 42] | S3-T30-VTM-<QP> |
| S3-T31-VTM | 8.2.3.4.2.2 | S3-R15 | VTM11.0 | VTM-02 | QP=[22, 27, 32, 37, 42] | S3-T31-VTM-<QP> |
| S3-T32-VTM | 8.2.3.4.2.2 | S3-R16 | VTM11.0 | VTM-02 | QP=[22, 27, 32, 37, 42] | S3-T32-VTM-<QP> |

##### 8.2.3.4.2	Configurations

###### 8.2.3.4.2.1	VTM-01: Main 10 Profile with no fixed Intra

To generate the anchor bitstreams, VTM Main 10 in low delay P configuration is used with screen content tools enabled. The settings are defined in the attached configuration file vtm-01.cfg. In addition, to enable screen content tools, the following settings are set to 1:  IBC, HashME, BDPCM.

In summary, each source sequence is encoded with the following changes:

-	QP: [22, 27, 32, 37, 42]

-	IntraPeriod with no fix interval

-	GOPSize is equal to 8. Each P picture refers to up to 4 preceding pictures in display order

-	IntraQPOffset is -1. P picture QP offsets are alternatively equal to 4, 5, and are set to 1 every 8 pictures.

###### 8.2.3.4.2.2	VTM-02: Main 10 Profile with fixed Intra every second

To generate the anchor bitstreams, VTM Main 10 in low delay P configuration is used with screen content tools enabled

The settings are defined in the attached configuration file vtm-02.cfg. In addition, to enable screen content tools, the following settings are set to 1:  IBC, HashME, BDPCM.

In summary, each source sequence is encoded with the following changes:

-	QP: [22, 27, 32, 37, 42]

-	IntraPeriod such that 1 second is achieved

-	DecodingRefreshType: (2) IDR

-	IntraQPOffset and QPoffsets are set equal to 0

-	Each P picture refers to immediately preceding pictures in display order.

##### 8.2.3.4.3	Test Results

VVC test streams are provided according to the key system here:

- https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Tests/VTM/Scenario-3

VVC test metrics are provided with the appropriate keys as defined in Table 8.2.3.4.1-1

- in the attached csv files

- https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Tests/VTM/Scenario-3/Metrics

Editor’s Note:

- Summary tables will be included.

- Please cross-check of the results latest until next SA4#113e meeting

#### 8.2.3.5	Scenario 4: Messaging and Social Sharing

#### 8.2.3.6	Scenario 5: Online Gaming

##### 8.2.3.6.1	Overview

Table 8.2.3.6.1-1 provides an overview of the H.266/VVC test tuples. Keys are identified to refer to the anchors in the context of the scenario.

The details are also provided here: http://dash.akamaized.net/WAVE/3GPP/5GVideo/Tests/VTM/Scenario-5/tests.csv.

Table 8.2.3.6.1-1 Test Tuple generation with H.266/VVC for Online Gaming Scenario

| Key | Clause | Reference Sequence | Reference Encoder | Configuration | Variations | Anchor Key |
| --- | --- | --- | --- | --- | --- | --- |
| S5-A01-VTM | 8.2.3.6.2.1 | S5-R01 | VTM11.0 | VTM-03 | QP=[22, 27, 32, 37] | S5-A01-VTM-<QP> |
| S5-A02-VTM | 8.2.3.6.2.1 | S5-R02 | VTM11.0 | VTM-03 | QP=[22, 27, 32, 37] | S5-A02-VTM-<QP> |
| S5-A03-VTM | 8.2.3.6.2.1 | S5-R03 | VTM11.0 | VTM-03 | QP=[22, 27, 32, 37] | S5-A03-VTM-<QP> |
| S5-A04-VTM | 8.2.3.6.2.1 | S5-R04 | VTM11.0 | VTM-03 | QP=[22, 27, 32, 37] | S5-A04-VTM-<QP> |
| S5-A05-VTM | 8.2.3.6.2.1 | S5-R05 | VTM11.0 | VTM-03 | QP=[22, 27, 32, 37] | S5-A05-VTM-<QP> |
| S5-A06-VTM | 8.2.3.6.2.1 | S5-R06 | VTM11.0 | VTM-03 | QP=[22, 27, 32, 37] | S5-A06-VTM-<QP> |
| S5-A07-VTM | 8.2.3.6.2.1 | S5-R07 | VTM11.0 | VTM-03 | QP=[22, 27, 32, 37] | S5-A07-VTM-<QP> |
| S5-A08-VTM | 8.2.3.6.2.1 | S5-R08 | VTM11.0 | VTM-03 | QP=[22, 27, 32, 37] | S5-A08-VTM-<QP> |
| S5-A09-VTM | 8.2.3.6.2.1 | S5-R09 | VTM11.0 | VTM-03 | QP=[22, 27, 32, 37] | S5-A09-VTM-<QP> |
| S5-A10-VTM | 8.2.3.6.2.1 | S5-R10 | VTM11.0 | VTM-03 | QP=[22, 27, 32, 37] | S5-A10-VTM-<QP> |
| S5-A11-VTM | 8.2.3.6.2.1 | S5-R11 | VTM11.0 | VTM-03 | QP=[22, 27, 32, 37] | S5-A11-VTM-<QP> |
| S5-A12-VTM | 8.2.3.6.2.1 | S5-R12 | VTM11.0 | VTM-03 | QP=[22, 27, 32, 37] | S5-A12-VTM-<QP> |
| S5-A13-VTM | 8.2.3.6.2.1 | S5-R13 | VTM11.0 | VTM-03 | QP=[22, 27, 32, 37] | S5-A13-VTM-<QP> |
| S5-A14-VTM | 8.2.3.6.2.2 | S5-R01 | VTM11.0 | VTM-04 | QP=[22, 27, 32, 37] | S5-A14-VTM-<QP> |
| S5-A15-VTM | 8.2.3.6.2.2 | S5-R02 | VTM11.0 | VTM-04 | QP=[22, 27, 32, 37] | S5-A15-VTM-<QP> |
| S5-A16-VTM | 8.2.3.6.2.2 | S5-R03 | VTM11.0 | VTM-04 | QP=[22, 27, 32, 37] | S5-A16-VTM-<QP> |
| S5-A17-VTM | 8.2.3.6.2.2 | S5-R04 | VTM11.0 | VTM-04 | QP=[22, 27, 32, 37] | S5-A17-VTM-<QP> |
| S5-A18-VTM | 8.2.3.6.2.2 | S5-R05 | VTM11.0 | VTM-04 | QP=[22, 27, 32, 37] | S5-A18-VTM-<QP> |
| S5-A19-VTM | 8.2.3.6.2.2 | S5-R06 | VTM11.0 | VTM-04 | QP=[22, 27, 32, 37] | S5-A19-VTM-<QP> |
| S5-A20-VTM | 8.2.3.6.2.2 | S5-R07 | VTM11.0 | VTM-04 | QP=[22, 27, 32, 37] | S5-A20-VTM-<QP> |
| S5-A21-VTM | 8.2.3.6.2.2 | S5-R08 | VTM11.0 | VTM-04 | QP=[22, 27, 32, 37] | S5-A21-VTM-<QP> |
| S5-A22-VTM | 8.2.3.6.2.2 | S5-R09 | VTM11.0 | VTM-04 | QP=[22, 27, 32, 37] | S5-A22-VTM-<QP> |
| S5-A23-VTM | 8.2.3.6.2.2 | S5-R10 | VTM11.0 | VTM-04 | QP=[22, 27, 32, 37] | S5-A23-VTM-<QP> |
| S5-A24-VTM | 8.2.3.6.2.2 | S5-R11 | VTM11.0 | VTM-04 | QP=[22, 27, 32, 37] | S5-A24-VTM-<QP> |
| S5-A25-VTM | 8.2.3.6.2.2 | S5-R12 | VTM11.0 | VTM-04 | QP=[22, 27, 32, 37] | S5-A25-VTM-<QP> |
| S5-A26-VTM | 8.2.3.6.2.2 | S5-R13 | VTM11.0 | VTM-04 | QP=[22, 27, 32, 37] | S5-A26-VTM-<QP> |

##### 8.2.3.6.2	Configurations

###### 8.2.3.6.2.1	VTM-03: Main 10 Profile with no fixed Intra

To generate the anchor bitstreams, VTM Main 10 in low delay B configuration is used with screen content tools enabled.

The settings are defined in the attached configuration file vtm-01.cfg. In addition, to enable screen content tools, the following settings are set to 1:  IBC, HashME, BDPCM.

In summary, each source sequence is encoded with the following changes:

-	QP: [22, 27, 32, 37, 42]

-	IntraPeriod with no fix interval

-	GOPSize is equal to 8. Each P picture refers to up to 4 preceding pictures in display order

-	IntraQPOffset is -1. P picture QP offsets are alternatively equal to 4, 5, and are set to 1 every 8 pictures.

###### 8.2.3.6.2.2	VTM-04: Main 10 Profile with fixed Intra every second

To generate the anchor bitstreams, VTM Main 10 in low delay B configuration is used with screen content tools enabled.

The settings are defined in the attached configuration file vtm-02.cfg. In addition, to enable screen content tools, the following settings are set to 1:  IBC, HashME, BDPCM.

In summary, each source sequence is encoded with the following changes:

-	QP: [22, 27, 32, 37, 42]

-	IntraPeriod such that 1 second is achieved

-	DecodingRefreshType: (2) IDR

-	IntraQPOffset and QPoffsets are set equal to 0

-	Each P picture refers to immediately preceding pictures in display order.

##### 8.2.3.6.3	Test Results

VVC test streams are provided according to the key system here:

- https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Tests/VTM/Scenario-5

VVC test metrics are provided with the appropriate keys as defined in Table 8.2.3.6.1-1

- in the attached csv files

- https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Tests/VTM/Scenario-5/Metrics

Editor’s Note:

- Summary tables will be included.

- Please cross-check of the results latest until next SA4#113e meeting

### 8.2.4	Characterization

tbd

## 8.3	Essential Video Coding (EVC)

### 8.3.1	Overview

The development of the MPEG-5 Essential Video Coding (EVC) standard is completed and its specification has been published in October 2020 as ISO/IEC 23094-1 [50].

The main goal of the EVC standard is to provide significantly improved compression capability over previous generation of video coding standards with timely publication of commercial terms. The EVC standard has been developed to provide a video codec for emerging delivery protocols and networks, such as 5G, enabling the delivery of high-quality video services to an ever-growing audience by providing improved coding performance.

The MPEG-5 EVC defines two important profiles, "Baseline" profile and "Main" profile. It was the design objective that the "Baseline" profile contains only technologies that are older than 20 years. The "Main" profile includes additional tools, each of which can be either cleanly disabled or switched to the corresponding baseline tool on an individual basis. Additionally, for still image coding, "Main Still Picture" and "Baseline Still Picture" profiles, which employ the same coding tools as in the corresponding video profiles, are defined.

MPEG has completed the EVC verification tests for HDR and SDR content with formal subjective testing showing that for equal subjective video quality, the EVC reference encoder using the EVC Main profile can achieve significant bit-rate reduction compared to the HEVC reference encoder using the HEVC Main 10 profile. These tests also demonstrated that the EVC reference encoder using the EVC Baseline profile can achieve similar bit-rate reduction compared to the AVC reference encoder using the AVC Progressive High 10 profile. Such analysis was performed using MOS BD-rate calculations.

The verification test conditions and results for SDR content are reported in [52]. The average bitrate savings for the EVC reference encoder using the EVC Main profile compared to the HEVC reference encoder using the HEVC Main 10 profile are approximately 39% for UHD SDR content, encoded using the random access configuration, and approximately 41% for HD SDR content, encoded using the low delay configuration. The average bit rate savings for the EVC reference encoder using the EVC Baseline profile compared to the AVC reference encoder using the AVC Progressive High 10 profile are approximately 39% for the UHD SDR content, encoded using the random access configuration, and approximately 34% for HD SDR content, encoded using the low delay configuration.

The verification test conditions and results for HDR content are reported in [53]. For random access configuration, the EVC reference encoder using the EVC Main profile provides around 36% bitrate reduction for HDR content at UHD resolution and 35% bitrate reduction for HDR content at HD resolution compared to the HEVC reference encoder with HDR/WCG encoder optimizations enabled, using the HEVC Main 10 profile, for equal subjective video quality.

Application areas especially targeted for the use of EVC include ultra-high definition 4K and 8K video, video with a high dynamic range and wide colour gamut, and video for immersive media applications such as 360° omnidirectional video, as well as conventional standard-definition and high-definition video content.

# 9	Gaps and Optimization Potential

## 9.1	Identified Gaps and Deficiencies with Existing Codecs

ffs

## 9.2	Potential Requirements for New Codecs

Ffs

# 10	Conclusions and Proposed Next Steps

Ffs

Annex A: Scenario Template

# A.1	Introduction

This annex provides a proposed template to introduce a Scenario for 5G Video. This template has been used to collect the scenarios in this report.

# A.2	Template

The following aspects are considered to be important for a scenario

1.	Scenario name <give the scenario a catchy name>

2.	Motivation for the scenario: Why is the scenario relevant for 5G and video? What is the expected traffic?

3.	Description of the scenario: This provides a description of the scenario addressing potentially the relation to a service 5G-based services and applications, including video formats (resolution, frame rates, color space, etc.), encoding and decoding requirements, adaptive streaming requirements, predominantly based on scenarios defined for 5G media streaming as well as for TR 26.925 and TR 26.928

4.	Supporting companies and 3GPP members:

a.	This documents the 3GPP members that support this scenario in terms of providing the information, test material, test requirements and the characterization for the tests. For each of the identified necessities, a tick box is is created in the template.

b.	Preferably several 3GPP members are included in the support, and in addition a video service provider may be included (not necessarily a 3GPP member).

c.	Cross-verification is preferably done by the supporters of the scenario

5.	Source format properties: This defines a clear range of the considered and relevant source formats, including the signal properties, but also the characteristics of the content. As an example, the source formats as defined in TS26.116 may be used which include:

a.	Spatial resolutions

b.	Chroma Format

c.	Chroma Subsampling

d.	Aspect ratios

e.	Frame rates

f.	Colour space formats

g.	Transfer Characteristics

h.	Bit depth

i.	Other signal properties

6.	Encoding and decoding constraints and settings: Typical encoding constraints and settings such as

a.	Relevant Codec and Codec Profile/Levels according to TS26.116 and TS26.511.

b.	Random access frequency

c.	Error resiliency requirements

d.	Bitrates and quality requirements

e.	Bitrate parameters (CBR, VBR, CAE, HRD parameters)

f.	ABR encoding requirements (switching frequency, etc.)

g.	Latency requirements and specific encoding settings

h.	Encoding context: real-time encoding, on device encoding, cloud-based encoding, offline encoding, etc.

i.	Required decoding capabilities

7.	Performance Metrics and Requirements:

a.	A clear definition on how the performance needs to be evaluated including metrics, etc addressing the main KPIs of the scenario.

b.	Objective measures such as PSNR, VMAF, etc, may be used.

c.	Subjective evaluation is not excluded and may be done, but needs commitment

8.	Interoperability Considerations for the application

a.	Streaming with DASH/HLS/CMAF

b.	RTP based delivery

9.	Test Sequences

a.	A set of selected test sequences that are provided by the proponents in order to do the evaluation. They should cover a set of source format properties

10.	Detailed test conditions:

a.	Provides a proposal for detailed test conditions, for example based on a reference software together with the sequences and configuration parameters.

11.	External Performance data

a.	References to external performance data that can be added, for example other SDOs, public documents and so on.

12.	Additional Information

Annex B: Data Formats and Metrics

# B.1	Introduction

This Annex provides a detailed overview on data formats and their usage for metrics computation.

# B.2	Raw Video Sequences

### B.2.1 Overview

For a raw video sequence used in the context of this Technical Report, the following metadata is expected to be provided:

1)	Spatial Resolution width

2)	Spatial Resolution height

3)	Chroma Format

4)	Chroma Sub-Sampling

5)	Picture Aspect ratio

6)	Frame rate

7) Bit depth

8) Color space format (default is 1 for BT.709)

9)	Transfer characteristics (default is 1 for BT.709)

10) Frame count (and optionally the duration)

In addition, the following aspects were collected that should be added:

11)	Video storage format

12)	MD5 of raw video

13)	Reference to bitstream (URL to weblink where the original sequence is available)

14)	Copyright statement

15)	Contact Person

16)	packing and storage format

17)	thumbnail preview

18)	mp4 preview

19)	Name of sequence

20)	Sequence key

21)	Some background information

Some information may be optional, others may be optionally defaulted.

A JSON scheme is defined in clause B.2.2 for this matter. An example is provided in clause B.2.3.

### B.2.2 JSON Schema

JSON schema for the raw format is here: https://github.com/haudiobe/5G-Video-Content/blob/main/3gpp-raw-schema.json.

Editor’s Note: this will be copied in here once agreed

### B.2.3 Example

{

"Sequence": {

"Name": "MovingText2-FullHD-420-10bit",

"Background": "The sequence MovingText2 is composed of several lines of text horizontally scrolling at different speeds over a gradation of background colours.",

"Scenario": "ScreenContent",

"Key": "S3-R3",

"URI": "./MovingText2-FullHD-420-10bit.yuv",

"md5": "ace7062d0cd680d212c698d91bee76d2",

"thumbnail": "./MovingText2.png",

"preview": "./MovingText2.mp4",

"TR26.955": "Annex C.6.2"

},

"Properties": {

"width": "1920",

"height": "1080",

"format": "yuv",

"packing": "planar",

"scan": "progressive",

"subsampling": "420",

"bitDepth": "10",

"frameRate": "60",

"colourPrimaries": "1",

"transferCharacteristics": "1",

"matrixCoefficients": "1",

"sampleAspectRatio": "1",

"duration": "10",

"frameCount": "600",

"startFrame": "1"

},

"copyRight": "The MovingText2 video sequence has been produced for the purpose of this study on the characterization of video codecs in 3GPP.",

"Contact": {

"Name": "Gilles Teniou",

"Company": "Tencent",

"e-mail": "teniou@tencent.com"

}

}

# B.3	Encoded Video Sequences

### B.2.1 Overview

For encoded video sequences used in the context of this Technical Report, the following metadata is expected to be provided:

1)	Explanation what this anchor is important for

2)	Reference sequence

3)	Reference encoder

4)	Encoder configuration

5)	Reference to bitstream

6) MD5 of bitstream

7) Metrics

8) MD5 of reconstructed sequence

9) Encoder log

10) Copyright statement

11) Contact Person

Some information may be optional, others may be optionally defaulted.

A JSON scheme is defined in clause B.2.2 for this matter. An example is provided in clause B.2.3.

### B.3.2 JSON Schema

JSON schema for the encoded format is here: https://github.com/haudiobe/5G-Video-Content/blob/main/3gpp-bin-schema.json.

Editor’s Note: this will be copied in here once agreed

### B.3.3 Example

{

"Bitstream": {

"URI": "./S3-A01-265_22.bin",

"key": "S3-A01-265_22",

"md5": "36d42a42c4f1096e6a6fe5d01cae9edf",

"size": 13709661

},

"Contact": {

"Company": "3GPP",

"e-mail": "3GPP_TSG_SA_WG4_VIDEO@LIST.ETSI.ORG"

},

"Generation": {

"config-file": "HM-01.cfg",

"encoder": "HM16.22",

"key": "S3-R01",

"log-file": "encoder_S3-A01-265_22.log",

"sequence": "MovingText2-4K-10bit.json",

"variant": "-qp 22"

},

"Metrics": {

"Bitrate": "10967.73",

"BitrateLog": "10940.37",

"DecodeTime": "119.08",

"EncodeTime": "51116.6",

"MS_SSIM": "0.9978",

"UPSNR": "52.96",

"VMAF": "100.0",

"VPSNR": "54.97",

"YPSNR": "53.44"

},

"Reconstruction": {

"decoder": "HM16.22",

"log-file": "decoder_S3-A01-265_22.log",

"md5": "unknown"

},

"Contact": {

"Verification": {

"date": "na",

"verified": "false"

},

"copyRight": "The MovingText2 video sequence has been produced for the purpose of this study on the characterization of video codecs in 3GPP."

}

Annex C: Candidate Reference Sequences

# C.1	Introduction

This annex provides a summary of candidate reference sequences that where discussed to be potentially suitable for one or multiple of the scenarios introduced in clause 6 of this Technical Report. For each candidate reference sequence, at least the following information is provided.

-	A summary of the sequence characteristics

-	A screenshot of the sequence

-	Source sequence properties

-	Information where the source sequence is hosted

-	Copyright and license information

The content is provided in JSON files here: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequence. The format of the reference sequences follows the proposed format in Annex B.

The sequences are summarized here: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/sequences.csv.

# C.2		Gaming Test Sequences

## C.2.1 	Overview

A set of gaming sequences is described in this sub-clause C.2.

Table C.2.1-1 proposes a summary of the gaming sequences according to the categories defined in section 6.6.3.

Table C.2.1-1, Gaming sequences categorization

| Sequence description |  |  | Categories |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Game sequence title | Game type | Sequences Characteristics | A | B: | C: | D: | E: |
| Arena of Valor (AOV) (Tencent) | multiplayer online battle arena | fix + Smooth travelling. Small incrustation. Stable at 90%. One pop-up incrustation. | X |  |  |  |  |
| Baolei-Man (Tencent) | online multiplayer first person shooter | Pursuit. Some irregular camera traveling. Small incrustation. Stable at 99% |  | X |  |  |  |
| Baloei-Balloon 4k (Tencent) | online multiplayer first person shooter | Pursuit. Some irregular camera traveling. Small incrustation. Stable at 90% |  | X |  |  |  |
| Baolei-Woman (Tencent) | online multiplayer first person shooter | Pursuit. Lots of irregular camera traveling. Small incrustation. Stable at 90% |  |  | X |  |  |
| Baolei-Yard 4K (Tencent) | online multiplayer first person shooter | Pursuit. Some irregular camera traveling. Small incrustation. Stable at 99% |  |  | X |  |  |
| Jianling-Temple (Tencent) | massive multiplayer online role-playing | Battle/pursuit. Lots of irregular camera traveling. Stable at 90% One small pop-up incrustation |  |  | X |  |  |
| Jianling-Beach (Tencent) | massive multiplayer online role-playing | Battle/pursuit. Dark scene. Irregular camera traveling. Small incrustation. Stable at 99% |  | X |  |  |  |
| Heroes of the Storm part 1 (kingston) | a crossover multiplayer online battle arena | Fix, plus 2 smooth small travelling | X |  |  |  |  |
| Project CARS part 1 (kingston) | Car racing game | Realistic rendering. Constant smooth movement. Small incrustation. Stable at 90% |  |  |  | X |  |
| WoW part 2 (kingston) | multiplayer online role play | Dark scene, irregular camera traveling. Small incrustation. Stable at 90% | X |  |  |  |  |
| Minecraft (twitch) | Casual game sandbox, brick construction | Graphic : Lots of irregular camera traveling. Very small incrustation. Stable at 100% | X |  |  |  |  |
| CS:GO (twitch) | a multiplayer first-person shooter | 1/ first half of sequence is dark (300pict) 2/ fade (15 pict) 3/ action : first-person shooter | X (first part) |  | X (3rd part) |  |  |
| StarCraft (twitch) | Real Time Strategy game | fix + smooth travelling Small incrustation stable at 90%. |  | X |  |  |  |

## C.2.2	AOV

### C.2.2.1	Introduction

Arena of Valor (AOV) ©Tencent Games – is a 3D MOBA (multiplayer online battle arena) game. Within a limited size map 2 groups of 5 vs 5 or 3 vs 3 players take part of a battle for which each player chooses a hero with particular power and abilities. The figure C.2.2-1 illustrates the gameplay of AOV with a screenshot of the AOV test sequence.

Figure C.2.2-1: Screenshot of AOV test sequence

### C.2.2.2	Sequence properties

The AOV test sequence properties are provided in Table C.2.2.2-1.

Table C.2.2.2-1 AOV sequence properties

| Parameter | Value |
| --- | --- |
| Resolution |  | 1920x1080 |  |
| Scan |  | Progressive |  |
| Frame Rate |  | 60 fps |  |
| Bit Depth |  | 8 |  |
| Length |  | 600 frames (10s) |  |
| YUV format |  | YUV 4:2:0 |  |
| Color components |  | Y’CbCr |  |
| Colour space |  | ITU-R BT.709 |  |

The sequence can be accessed here:

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/AOV/AOV.json

### C.2.2.3	Copyright and license information

The AOV video sequence is made available under the following copyright disclaimer.

Copyright © 1998 - 2018 Tencent. All Rights Reserved.

This sequence and all intellectual property rights therein remain the property of Shenzhen Tencent Computer System Co. Ltd. and Tencent America LLC. This sequence may only be used for the purpose of developing, testing and promulgating technology standards (the Purpose). Shenzhen Tencent Computer System Co. Ltd. and Tencent America LLC. make no warranties with respect to this sequence and expressly disclaim any warranties regarding its fitness for any purpose. This disclaimer shall remain attached to this sequence except when this sequence is used for the Purpose.

## C.2.3	Baolei-Man

### C.2.3.1	Introduction

The Baolei-Man test sequence is an extract from a famous online multiplayer game in which many players play in a battle royale mode meaning that in survival mode, the last standing one is the winner. The figure C.2.3-1 shows a screenshot of the Baolei-Man test sequence.

Figure C.2.3-1: Screenshot of Baolei-Man test sequence

### C.2.3.2	Sequence properties

The Baolei-Man test sequence is available in fullHD with the following properties specified hereafter in Table C.2.3.2-1.

Table C.2.3.2-1 Baolei-Man sequence properties

| Parameter | Value fullHD |
| --- | --- |
| Resolution |  | 1920x1080 |  |
| Scan |  | Progressive |  |
| Frame Rate |  | 60 fps |  |
| Bit Depth |  | 8 |  |
| Length |  | 600 frames (10s) |  |
| YUV format |  | YUV 4:2:0 |  |
| Color components |  | Y’CbCr |  |
| Colour space |  | ITU-R BT.709 |  |

The sequence can be accessed here:

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/Baolei-Man/baolei-man.json

### C.2.3.3	Copyright and license information

The Baolei-Man video sequence is made available under the following copyright disclaimer.

Copyright © 1998 - 2020 Tencent. All Rights Reserved.

This sequence and all intellectual property rights therein remain the property of Shenzhen Tencent Computer System Co. Ltd.. This sequence may only be used for the purpose of developing, testing and promulgating technology standards (the Purpose). Shenzhen Tencent Computer System Co. Ltd. make no warranties with respect to this sequence and expressly disclaim any warranties regarding its fitness for any purpose. This disclaimer shall remain attached to this sequence except when this sequence is used for the Purpose.

## C.2.4	Baolei-Woman

### C.2.4.1	Introduction

The Baolei-Woman test sequence is also an extract from the same game as for Balei-Man. The figure C.2.4-1 shows a screenshot of the Baolei-Woman test sequence.

Figure C.2.4-1: Screenshot of Baolei-Woman test sequence

### C.2.4.2	Sequence properties

Similarly to the Baolei-Man test sequence, Baolei-Woman is also available in FullHD with the following properties specified hereafter in Table C.2.4.2-1.

Table C.2.4.2-1 Baolei-Woman sequences properties

| Parameter | Value FullHD |
| --- | --- |
| Resolution |  | 1920x1080 |  |
| Scan |  | Progressive |  |
| Frame Rate |  | 60 fps |  |
| Bit Depth |  | 8 |  |
| Length |  | 600 frames (10s) |  |
| YUV format |  | YUV 4:2:0 |  |
| Color components |  | Y’CbCr |  |
| Colour space |  | ITU-R BT.709 |  |

The sequence can be accessed here:

https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/Baolei-Woman/baolei-woman.json

### C.2.4.3	Copyright and license information

The Baolei-Woman video sequence is made available under the following copyright disclaimer.

Copyright © 1998 - 2020 Tencent. All Rights Reserved.

This sequence and all intellectual property rights therein remain the property of Shenzhen Tencent Computer System Co. Ltd.. This sequence may only be used for the purpose of developing, testing and promulgating technology standards (the Purpose). Shenzhen Tencent Computer System Co. Ltd. make no warranties with respect to this sequence and expressly disclaim any warranties regarding its fitness for any purpose. This disclaimer shall remain attached to this sequence except when this sequence is used for the Purpose.

## C.2.5	Baolei-Balloon

### C.2.5.1	Introduction

The Baolei-Balloon test sequence is also an extract from the same game as for Balei-Man. The figure C.2.5-1 shows a screenshot of the Baolei-Balloon test sequence.

Figure C.2.5-1: Screenshot of Baolei-Balloon test sequence

### C.2.5.2	Sequence properties

The Baolei-Balloon test sequence is available in 4K with the following properties specified hereafter in Table C.2.5.2-1.

Table C.2.5.2-1 Baolei-Balloon sequence properties

| Parameter | Value 4K |
| --- | --- |
| Resolution |  | 4096x2160 |  |
| Scan |  | Progressive |  |
| Frame Rate |  | 60 fps |  |
| Bit Depth |  | 8 |  |
| Length |  | 600 frames (10s) |  |
| YUV format |  | YUV 4:2:0 |  |
| Color components |  | Y’CbCr |  |
| Colour space |  | ITU-R BT.709 |  |

The sequence can be accessed here:

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/Baolei-Balloon/baolei-balloon.json

### C.2.5.3	Copyright and license information

The Baolei-Balloon video sequence is made available under the following copyright disclaimer.

Copyright © 1998 - 2020 Tencent. All Rights Reserved.

This sequence and all intellectual property rights therein remain the property of Shenzhen Tencent Computer System Co. Ltd.. This sequence may only be used for the purpose of developing, testing and promulgating technology standards (the Purpose). Shenzhen Tencent Computer System Co. Ltd. make no warranties with respect to this sequence and expressly disclaim any warranties regarding its fitness for any purpose. This disclaimer shall remain attached to this sequence except when this sequence is used for the Purpose.

## C.2.6	Baolei-Yard

### C.2.6.1	Introduction

The Baolei-Yard test sequence is also an extract from the same game as for Balei-Man. The figure C.2.6-1 shows a screenshot of the Baolei-Yard test sequence.

Figure C.2.6-1: Screenshot of Baolei-Yard test sequence

### C.2.6.2	Sequence properties

Similarly to the Baolei-Balloon test sequence, Baolei-Yard is also available in 4K with the following properties specified hereafter in Table C.2.6.2-1.

Table C.2.4.2-1 Baolei-Yard sequences properties

| Parameter | Value 4K |
| --- | --- |
| Resolution |  | 4096x2160 |  |
| Scan |  | Progressive |  |
| Frame Rate |  | 60 fps |  |
| Bit Depth |  | 8 |  |
| Length |  | 600 frames (10s) |  |
| YUV format |  | YUV 4:2:0 |  |
| Color components |  | Y’CbCr |  |
| Colour space |  | ITU-R BT.709 |  |

The sequence can be accessed

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/Baolei-Yard/Baolei-Yard.json

### C.2.6.3	Copyright and license information

The Baolei-Yard video sequence is made available under the following copyright disclaimer.

Copyright © 1998 - 2020 Tencent. All Rights Reserved.

This sequence and all intellectual property rights therein remain the property of Shenzhen Tencent Computer System Co. Ltd.. This sequence may only be used for the purpose of developing, testing and promulgating technology standards (the Purpose). Shenzhen Tencent Computer System Co. Ltd. make no warranties with respect to this sequence and expressly disclaim any warranties regarding its fitness for any purpose. This disclaimer shall remain attached to this sequence except when this sequence is used for the Purpose.

## C.2.7	Jianling-Temple

### C.2.7.1	Introduction

The Jianling-Temple test sequence is an extract from another famous massive multiplayer online role-playing game (MMORPG) having two gameplay modes Player-versus-Environment (PvE) and Player-versus-player (PvP). The figure C.2.7-1 is a screenshot of the Jianling-Temple test sequence.

Figure C.2.7-1: Screenshot of Jianling-Temple test sequence

### C.2.7.2	Sequence properties

The Jianling-Temple test sequence properties are provided in Table C.2.7.2-1.

Table C.2.7.2-1 Jianling-Temple sequence properties

| Parameter | Value |
| --- | --- |
| Resolution |  | 1920x1080 |  |
| Scan |  | Progressive |  |
| Frame Rate |  | 60 fps |  |
| Bit Depth |  | 8 |  |
| Length |  | 600 frames (10s) |  |
| YUV format |  | YUV 4:2:0 |  |
| Color components |  | Y’CbCr |  |
| Colour space |  | ITU-R BT.709 |  |

The sequence can be accessed here:

- https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/Jianling-Temple/jianling-temple.json

### C.2.7.3	Copyright and license information

The Jianling-Temple video sequence is made available under the following copyright disclaimer.

Copyright © 1998 - 2020 Tencent & NCSOFT. All Rights Reserved.

This sequence and all intellectual property rights therein remain the property of Shenzhen Tencent Computer System Co. Ltd. and NCSOFT Co.Ltd.. This sequence may only be used for the purpose of developing, testing and promulgating technology standards (the Purpose). Shenzhen Tencent Computer System Co. Ltd. and NCSOFT Co.Ltd. make no warranties with respect to this sequence and expressly disclaim any warranties regarding its fitness for any purpose. This disclaimer shall remain attached to this sequence except when this sequence is used for the Purpose.

## C.2.8	Jianling-Beach

### C.2.8.1	Introduction

The Jianling-Beach test sequence is another extract from the same MMORPG game as the Jianling-Temple sequence. The figure C.2.8-1 is a screenshot of the Jianling-Beach test sequence.

Figure C.2.8-1: Screenshot of Jianling-Beach test sequence

### C.2.8.2	Sequence properties

The Jianling-Beach test sequence properties are provided in Table C.2.8.2-1.

Table C.2.8.2-1 Jianling-Beach sequence properties

| Parameter | Value |
| --- | --- |
| Resolution |  | 1920x1080 |  |
| Scan |  | Progressive |  |
| Frame Rate |  | 60 fps |  |
| Bit Depth |  | 8 |  |
| Length |  | 600 frames (10s) |  |
| YUV format |  | YUV 4:2:0 |  |
| Color components |  | Y’CbCr |  |
| Colour space |  | ITU-R BT.709 |  |

The sequence can be accessed

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/Jianling-Beach/jianling-beach.json

### C.2.8.3	Copyright and license information

The Jianling-Beach video sequence is made available under the following copyright disclaimer.

Copyright © 1998 - 2020 Tencent & NCSOFT. All Rights Reserved.

This sequence and all intellectual property rights therein remain the property of Shenzhen Tencent Computer System Co. Ltd. and NCSOFT Co.Ltd.. This sequence may only be used for the purpose of developing, testing and promulgating technology standards (the Purpose). Shenzhen Tencent Computer System Co. Ltd. and NCSOFT Co.Ltd. make no warranties with respect to this sequence and expressly disclaim any warranties regarding its fitness for any purpose. This disclaimer shall remain attached to this sequence except when this sequence is used for the Purpose.

## C.2.9	Heroes of the Storm

### C.2.9.1	Introduction

Heroes of the Storm is a crossover multiplayer online battle arena video game developed and published by Blizzard Entertainment. The figure C.2.9.1-1 illustrates the gameplay of Heroes of the Storm with a screenshot of the test sequence provided by Kingston University, London and Technical University, Berlin. [42] [43].

Figure C.2.9.1-1: Screenshot of Heroes of the Storm test sequence

### C.2.9.2	Sequence properties

The Heroes of the Storm test sequence properties are provided in Table C.2.9.2-1.

Table C.2.2.2-1 Heroes of the Storm sequence properties

| Parameter | Value |
| --- | --- |
| Resolution |  | 1920x1080 |  |
| Scan |  | Progressive |  |
| Frame Rate |  | 30 fps |  |
| Bit Depth |  | 8 |  |
| Length |  | 900 frames (30s), but only first 300 frames to be used |  |
| YUV format |  | YUV 4:2:0 |  |
| Color components |  | Y’CbCr |  |
| Colour space |  | ITU-R BT.709 |  |

The Heroes of the Storm video sequence part 1 provided by Kingston University, London and Technical University, Berlin is available at this location:

-	https://kingston.app.box.com/v/GamingVideoSET. Part 2.

The sequence can be accessed

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/Heroes-of-the-Storm/Heroes-of-the-Storm.json

### C.2.9.3	Copyright and license information

The Heroes of the Storm video sequence provided by Kingston University, London and Technical University, Berlin is made available under the following copyright disclaimer.

https://kingston.app.box.com/v/GamingVideoSET/file/289578680253

## C.2.10	Project CARS

### C.2.10.1	Introduction

Project CARS is a racing video game series developed by Slightly Mad Studios and published by Bandai Namco Entertainment. The figure C.2.10.1-1 illustrates the gameplay of Project CARS with a screenshot of the test sequence provided by Kingston University, London and Technical University, Berlin. [42] [43].

Figure C.2.10.1-1: Screenshot of Project CARS test sequence

### C.2.10.2	Sequence properties

The Project CARS test sequence properties are provided in Table C.2.10.2-1.

Table C.2.10.2-1 Project CARS sequence properties

| Parameter | Value |
| --- | --- |
| Resolution |  | 1920x1080 |  |
| Scan |  | Progressive |  |
| Frame Rate |  | 30 fps |  |
| Bit Depth |  | 8 |  |
| Length |  | 900 frames (30s), but only first 300 frames to be used |  |
| YUV format |  | YUV 4:2:0 |  |
| Color components |  | Y’CbCr |  |
| Colour space |  | ITU-R BT.709 |  |

The Project CARS video sequence part 1 provided by Kingston University, London and Technical University, Berlin is available at this location:

https://kingston.app.box.com/v/GamingVideoSET. Part 2

The sequence can be accessed

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/Project-CARS/Project-CARS.json

### C.2.10.3	Copyright and license information

The Project CARS video sequence provided by Kingston University, London and Technical University, Berlin is made available under the following copyright disclaimer:

https://kingston.app.box.com/v/GamingVideoSET/file/289578680253

## C.2.11	World of WarCraft: WoW

### C.2.11.1	Introduction

WoW is a multiplayer online role play developed and published by Blizzard Entertainment where players control a character avatar within a game world in third- or first-person view, exploring the landscape, fighting various monsters, completing quests, and interacting with non-player characters or other players. The figure C.2.11.1-1 illustrates the gameplay of WoW with a screenshot of the test sequence, which is a dark scene, provided by Kingston University, London and Technical University, Berlin. [42] [43].

Figure C.2.11.1-1: Screenshot of World of WarCraft: WoW test sequence

### C.2.11.2	Sequence properties

The WoW test sequence part 2, provided by Kingston University, London and Technical University, Berlin, sequence properties are provided in Table C.2.11.2-1.

Table C.2.11.2-1 World of WarCraft sequence properties

| Parameter | Value |
| --- | --- |
| Resolution |  | 1920x1080 |  |
| Scan |  | Progressive |  |
| Frame Rate |  | 30 fps |  |
| Bit Depth |  | 8 |  |
| Length |  | 900 frames (30s), but only first 300 frames to be used |  |
| YUV format |  | YUV 4:2:0 |  |
| Color components |  | Y’CbCr |  |
| Colour space |  | ITU-R BT.709 |  |

The WoW video sequence part 2 provided by Kingston University, London and Technical University, Berlin is available at this location:

https://kingston.app.box.com/v/GamingVideoSET. Part 2

The sequence can be accessed

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/World-of-WarCraft/World-of-WarCraft.json

### C.2.11.3	Copyright and license information

The WoW video sequence provided by Kingston University, London and Technical University, Berlin is made available under the following copyright disclaimer:

https://kingston.app.box.com/v/GamingVideoSET/file/289578680253

## C.2.12	Minecraft

### C.2.12.1	Introduction

The Minecraft test sequence is an extract from a famous a sandbox video game developed by Mojang Studios in which players explore a blocky, procedurally-generated 3D world with infinite terrain, and may discover and extract raw materials, craft tools and items, and build structures or earthworks. The figure C.2.12.1-1 shows a screenshot of the Minecraft test sequence provided by DERF_TWITCH.

Figure C.2.12.1-1: Screenshot of Minecraft test sequence

### C.2.12.2	Sequence properties

The Minecraft test sequence properties are provided in Table C.2.12.2-1.

Table C.2.12.2-1 Minecraft sequence properties

| Parameter | Value |
| --- | --- |
| Resolution |  | 1920x1080 |  |
| Scan |  | Progressive |  |
| Frame Rate |  | 60 fps |  |
| Bit Depth |  | 8 |  |
| Length |  | 600 frames (10s) |  |
| YUV format |  | YUV 4:2:0 |  |
| Color components |  | Y’CbCr |  |
| Colour space |  | ITU-R BT.709 |  |

The Minecraft sequence provided by DERTF-Twitch can be downloaded in y4m format at following URL: https://media.xiph.org/video/derf/twitch/y4m/ and can be converted into YUV format with ffmpeg by using the following exemplary command line: ffmpeg -i Minecraft.y4m Minecraft.yuv.

The sequence can be accessed

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/MineCraft/MineCraft.json

### C.2.12.3	Copyright and license information

The copyright under which the Minecraft video sequence from DERF_Twitch is made available on that link:

https://media.xiph.org/video/derf/twitch/copyright.txt

## C.2.13	CS:GO

### C.2.13.1	Introduction

The CS:GO test sequence from Twitch is an extract from a is a multiplayer first-person shooter video game developed by Valve and Hidden Path Entertainment where two opposing teams, known as the Terrorists and the Counter-Terrorists, compete in game modes to complete objectives, such as securing a location to plant or defuse a bomb and rescuing or guarding hostages. The figure C.2.13.1-1 shows a screenshot of the CS:GO test sequence provided by DERF-TWITCH.

Figure C.2.11.1-1: Screenshot of CS:GO test sequence

### C.2.13.2	Sequence properties

The CS:GO test sequence properties are provided in Table C.2.13.2-1.

Table C.2.13.2-1 CS:GO sequence properties

| Parameter | Value |
| --- | --- |
| Resolution |  | 1920x1080 |  |
| Scan |  | Progressive |  |
| Frame Rate |  | 60 fps |  |
| Bit Depth |  | 8 |  |
| Length |  | 600 frames (10s) |  |
| YUV format |  | YUV 4:2:0 |  |
| Color components |  | Y’CbCr |  |
| Colour space |  | ITU-R BT.709 |  |

The CS:GO sequence provided by DERF-Twitch can be downloaded in y4m format at following URL: https://media.xiph.org/video/derf/twitch/y4m/  and can be converted into YUV format with ffmpeg by using the following exemplary command line: ffmpeg -i CSGO.y4m CSGO.yuv.

The sequence can be accessed

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/CS-GO/CS-GO.json

### C.2.13.3	Copyright and license information

The copyright under which the CS:GO video sequence provided by DERF-Twitch is made available can be found on that link:

https://media.xiph.org/video/derf/twitch/copyright.txt.

## C.2.14	StarCraft

### C.2.14.1	Introduction

The StarCraft test sequence from Twitch is an extract from a famous Real Time Strategy game developed and published by Blizzard Entertainment where players enter a tournament of 5 games and chose to play one of three alien categories with distinct characteristic and abilities. The figure C.2.12.1-1 shows a screenshot of the StarCraft test sequence provided by DERF-Twitch.

Figure C.2.14.1-1: Screenshot of StarCraft test sequence

### C.2.14.2	Sequence properties

The StarCraft test sequence properties are provided in Table C.2.14.2-1.

Table C.2.14.2-1 StarCraft sequence properties

| Parameter | Value |
| --- | --- |
| Resolution |  | 1920x1080 |  |
| Scan |  | Progressive |  |
| Frame Rate |  | 60 fps |  |
| Bit Depth |  | 8 |  |
| Length |  | 600 frames (10s) |  |
| YUV format |  | YUV 4:2:0 |  |
| Color components |  | Y’CbCr |  |
| Colour space |  | ITU-R BT.709 |  |

The StarCraft sequence provided by Derf-Twitch can be downloaded in y4m format at following URL: https://media.xiph.org/video/derf/twitch/y4m/ and can be converted into YUV format with ffmpeg by using the following exemplary command line: ffmpeg -i StartCraft.y4m Startcraft.yuv.

The sequence can be accessed

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/StarCraft/StarCraft.json

### C.2.14.3	Copyright and license information

The copyright under which the StarCraft video sequence provided by DERF-Twitch is made available can be found on that link: https://media.xiph.org/video/derf/twitch/copyright.txt.

# C.3	4K-TV Sequences

## C.3.1 	SDR Category

### C.3.1.1	Candidate Test Sequences

#### C.3.1.1.1	Introduction

In this Annex, a wide range of SDR test material is reported. The candidate sequences are extracted from test material used in video processing community. The following sections describe the test material in terms of format characteristics and copyright.

#### C.3.1.1.2	Ultra Video Group Sequences

The Ultra Video Group (UVG) sequences are publicly available and can be used under CC BY-NC 3.0 license. The sequences are available for download at https://github.com/ultravideo/UVG-4K-Dataset and an overview is provided in Table C.3.1.1.2-1.

Table C.3.1.1.2-1: UVG test material description

| Name | Resolution | Frame Rate | Colour Gamut | Duration |
| --- | --- | --- | --- | --- |
| CityAlley | 3840 x 2160 | 50 | BT.709 | 12s |
| FlowerFocus | 3840 x 2160 | 50 | BT.709 | 12s |
| FlowerKids | 3840 x 2160 | 50 | BT.709 | 12s |
| FlowerPan | 3840 x 2160 | 50 | BT.709 | 12s |
| RaceNight | 3840 x 2160 | 50 | BT.709 | 12s |
| RiverBank | 3840 x 2160 | 50 | BT.709 | 12s |
| SunBath | 3840 x 2160 | 50 | BT.709 | 6s |
| Twilight | 3840 x 2160 | 50 | BT.709 | 12s |

#### C.3.1.1.3	JVET Sequences

The JVET test material for UHD SDR sequences is composed by six sequences described in Table C.3.1.1.3-1. The licensing information referenced in the table is described below:

1.	CC BY-NC-ND 4.0

2.	Our dataset is available for free and academic research only, no commercial use. Please cite our paper (L. Song, X. Tang, W. Zhang, X. Yang, P. Xia, The SJTU 4K Video Sequence Dataset, the Fifth International Workshop on Quality of Multimedia Experience (QoMEX2013), Klagenfurt, Austria, July 3rd-5th, 2013) in any published work if you use those video sequences.

3.	Please note that this test material remains the property of the Institute of Research & Technology b<>com and is solely provided “as is" to ITU-T SG16 Q6 and ISO/IEC JTC1/SC29/WG11, on a non-transferable and non-exclusive basis, and solely for the purpose of development, testing and promulgation of video coding standards. Any disclosure to third parties, exhibition, broadcast, distribution, public display or other exploitation of the test material is strictly prohibited.

4.	The proposed test sequences in this document and all intellectual property rights therein remain the property of Huawei Technologies Co., Ltd., and National Engineering Research Center.

Table C.3.1.1.3-1: JVET test material description

| Name | Resolution | Frame Rate | Colour Gamut | Number of frames | License |
| --- | --- | --- | --- | --- | --- |
| Tango2 | 3840 x 2160 | 60 | BT.709 | 294 | See 1. |
| FoodMarket4 | 3840 x 2160 | 60 | BT.709 | 300 | See 1. |
| Campfire | 3840 x 2160 | 30 | BT.709 | 300 | See 2. |
| Catrobots1 | 3840 x 2160 | 60 | BT.709 | 300 | See 3. |
| Daylightroad2 | 3840 x 2160 | 60 | BT.709 | 300 | See 4. |
| Parkrunning3 | 3840 x 2160 | 50 | BT.709 | 300 | See 4. |

#### C.3.1.1.4	Netflix Sequences

The Netflix dataset is composed by eighteen 4K sequences described in Table C.3.1.1.4-1. The sequences can be used under the CC BY-NC-ND 4.0 license restriction.

Table C.3.1.1.4-1: Netflix test material description

| Name | Resolution | Frame Rate | Colour Gamut | Number of frames |
| --- | --- | --- | --- | --- |
| Netflix_Aerial | 4096 x 2160 | 59.94 | BT.709 | 1199 |
| Netflix_DinnerScene | 4096 x 2160 | 59.94 | BT.709 | 1199 |
| Netflix_RollerCoaster | 4096 x 2160 | 59.94 | BT.709 | 1199 |
| Netflix_DrivingPOV | 4096 x 2160 | 59.94 | BT.709 | 1199 |
| Netflix_BarScene | 4096 x 2160 | 59.94 | BT.709 | 1199 |
| Netflix_ToddlerFountain | 4096 x 2160 | 59.94 | BT.709 | 1199 |
| Netflix_Dancers | 4096 x 2160 | 59.94 | BT.709 | 1199 |
| Netflix_PierSeaside | 4096 x 2160 | 59.94 | BT.709 | 1199 |
| Netflix_WindAndNature | 4096 x 2160 | 59.94 | BT.709 | 1199 |
| Netflix_Boat | 4096 x 2160 | 59.94 | BT.709 | 300 |
| Netflix_FoodMarket | 4096 x 2160 | 59.94 | BT.709 | 600 |
| Netflix_Tango | 4096 x 2160 | 59.94 | BT.709 | 294 |
| Netflix_BoxingPractice | 4096 x 2160 | 59.94 | BT.709 | 254 |
| Netflix_Narrator | 4096 x 2160 | 59.94 | BT.709 | 300 |
| Netflix_TunnelFlag | 4096 x 2160 | 59.94 | BT.709 | 600 |
| Netflix_RitualDance | 4096 x 2160 | 59.94 | BT.709 | 600 |
| Netflix_FoodMarket2 | 4096 x 2160 | 59.94 | BT.709 | 300 |
| Netflix_SquareAndTimelapse | 4096 x 2160 | 59.94 | BT.709 | 600 |
| Netflix_Aerial | 4096 x 2160 | 59.94 | BT.709 | 1199 |
| Netflix_DinnerScene | 4096 x 2160 | 59.94 | BT.709 | 1199 |
| Netflix_RollerCoaster | 4096 x 2160 | 59.94 | BT.709 | 1199 |

#### C.3.1.1.5	EBU Sequences

The EBU dataset is composed by two sequences described in Table C.3.1.1.5-1. The sequences can be used under the CC BY-NC-ND 4.0 license restriction.

Table C.3.1.1.5-1: EBU test material description

| Name | Resolution | Frame Rate | Colour Gamut | Number of frames |
| --- | --- | --- | --- | --- |
| Rain_fruits | 3840 x 2160 | 50 | BT.709 | 600 |
| Lupo_candlelight | 3840 x 2160 | 50 | BT.709 | 600 |

#### C.3.1.1.6	SVT Sequences

The SVT dataset is composed by five sequences described in Table C.3.1.1.6-1. The sequences can be used under the license available at https://tech.ebu.ch/webdav/site/tech/shared/hdtv/svt-multiformat-conditions-v10.pdf .

Table C.3.1.1.6-1: SVT test material description

| Name | Resolution | Frame rate | Colour Gamut | Number of frames |
| --- | --- | --- | --- | --- |
| CrowdRun | 3840 x 2160 | 50 | BT.709 | 500 |
| ParkJoy | 3840 x 2160 | 50 | BT.709 | 500 |
| DucksTakeOff | 3840 x 2160 | 50 | BT.709 | 500 |
| IntoTree | 3840 x 2160 | 50 | BT.709 | 500 |
| OldTownCross | 3840 x 2160 | 50 | BT.709 | 500 |

#### C.3.1.1.7	4ever Sequences

The 4ever dataset is composed by two sequences described in Table C.3.1.1.7-1. The sequences can be used under the license described below:

The proposed test sequences Brest_Sedof_3840x2160p60_10b.yuv and Paris_Montmartre_3840x2160p60_10b, and all intellectual property rights therein remain the property of 4EVER consortium partners: Orange, AMP Visual TV, ATEME, France Télévisions, GlobeCast, InterDigital, Highlands Technologies Solutions, INSA Rennes, TeamCast and Télécom ParisTech

- The following uses are allowed for the proposed sequences:

-	Can be published in technical papers, played at technology research and development events.

-	Can be used in 3GPP SA4.

-	The following use is not allowed for the proposed sequences:

-	Do not publish snapshots in product brochures.

-	Do not redistribute video with a commercial product.

Table C.3.1.1.7-1: 4ever test material description

| Name | Resolution | Frame Rate | Colour Gamut | Number of frames |
| --- | --- | --- | --- | --- |
| 4EVER_Brest_Sedof | 3840 x 2160 | 60 | BT.2020 | 600 |
| 4EVER_Paris_Montmartre | 3840 x 2160 | 60 | BT.2020 | 600 |

#### C.3.1.1.8	CableLabs Sequences

The CableLabs dataset is composed by eight sequences described in the Table C.3.1.1.8-1. The sequences can be used under the CC BY-NC-ND 3.0 license restriction.

Table C.3.1.1.8-1: CableLabs test material description

| Name | Resolution | Frame Rate | Colour Gamut | Duration | Comment on selected intervals |
| --- | --- | --- | --- | --- | --- |
| Lifting Off | 3840 x 2160 | 24/1.001 | BT.709 | 3:21 | Nice colored pictures, som burned in text, scene cuts (2) |
| Seconds that count | 3840 x 2160 | 24/1.001 | BT.709 | 5:22 | Not critical |
| Unspoken Friend | 3840 x 2160 | 24/1.001 | BT.709 | 4:17 | Not critical |
| Indoor Soccer | 3840 x 2160 | 24/1.001 | BT.709 | 4:12 | Fast motion, scene cuts (4) |
| Skateboarding | 3840 x 2160 | 24/1.001 | BT.709 | 4:36 | Not critical |
| Eldorado | 3840 x 2160 | 24/1.001 | BT.709 | 3:02 | Panning up/down on a tree (oak), no scene cut |
| Moment of Intensity | 3840 x 2160 | 60/1.001 | BT.709 | 3:10 | Skate board, no scene cuts |
| Ancient Thought | 3840 x 2160 | 24/1.001 | BT.709 | 3:08 | Not critical |

All of the sequences are not of interest for compression evaluation. A selection of some critical time intervals have been conducted based on experience on compression. Especially intervals with fast or complex motion, some including scene cuts, representative of TV contents have been selected. Static pictures, or pictures with artistic intent with few spatial details have been discarded. To remain in the scope of the scenario, the following sub sequences have been selected as final candidates:

- Lifting off: from 00:05 to 00:22

- Indoor Soccer: from 01:06 to 01:23

- Eldorado: from 01:33 to 01:50

- Life Untouched: from 00:00 to 00:22

The extraction of subpart from original ProRes sequences is realized with FFmpeg release 4.3.1 using the following command line:

./ffmpeg.exe -i $IN.mov -vcodec rawvideo -ss $START_TIME -t $DURATION -pix_fmt yuv420p10le $OUT.yuv

Where $IN and $OUT are respectively the input and output sequences, $START_TIME and $DURATION respectively the start time and duration in hh:mm:ss format.

### C.3.1.2	Selected Test Sequences

#### C.3.1.2.1	Introduction

In order to decide which sequences should be used in this specification, the following process has been used:

1.	Run informative encoding using x265 on the candidate sequences

2.	Run SI-TI metrics computation on the candidate sequences

3.	Select two diverse datasets based on R-D and SI-TI information.

#### C.3.1.2.2	x265 Encoding

To represent candidate sequences in the R-D plane, fixed QP encodings are carried out for QPs [12, 17, 22, 27, 32, 37, 42] using ffmpeg release 4.3.1, with the following command line:

./ffmpeg.exe -pix_fmt yuv420p10le -s $Wx$H -framerate $FR -i $IN -vcodec libx265 -qp $QP -preset fast -x265-params “psnr=1:ssim=1” $OUT.mp4

Where $W and $H are respectively the sequence width and height, $FR the sequence frame rate, $QP the selected quantization parameter. $IN and $OUT are respectively the input and output files.

From the obtained log, the overall bitrate and PSNR are extracted to be used for plotting. The encoding result are presented below:

- Figure 3.1.2.2-1 provides the R-D curves for the UVG dataset,

- Figure 3.1.2.2-2 provides the R-D curves for the JVET dataset,

- Figure 3.1.2.2-3 provides the R-D curves for the Netflix dataset,

- Figure 3.1.2.2-4 provides the R-D curves for the EBU, SVT and 4ever dataset,

- Figure 3.1.2.2-5 provides the R-D curves for the CableLabs dataset.

All sequences being represented in the targeted [10-40] Mbps range (red box).

Figure C. 3.1.2.2-1: R-D curves for the UVG dataset

Figure C. 3.1.2.2-2: R-D curves for the JVET dataset

Figure C. 3.1.2.2-3: R-D curves for the Netflix dataset

Figure C. 3.1.2.2-4: R-D curves for the EBU, SVT and 4ever datasets

Figure C. 3.1.2.2-5: R-D curves for the CableLabs dataset

#### C.3.1.2.3	SI-TI Metrics

The analysis is based on spatial perceptual information (SI) and temporal perceptual information (TI) metrics calculated as defined in ITU-P.910 recommendation [40]. In order to reduce the bias introduced by scene-cuts in TI metric computation, the TI at scene-cut boundaries are removed from computation.

From the obtained log, the SI and TI metrics are presented below:

- Figure 3.1.2.3-1 provides the SI-TI plots for the UVG dataset,

- Figure 3.1.2.3-2 provides the SI-TI plots for the JVET dataset,

- Figure 3.1.2.3-3 provides the SI-TI plots for the Netflix dataset,

- Figure 3.1.2.3-4 provides the SI-TI plots for the EBU, SVT, 4ever and CableLabs dataset.

Figure C.3.1.2.3-1: SI-TI for the UVG dataset

Figure C.3.1.2.3-2: SI-TI for the JVET dataset

Figure C.3.1.2.3-3: SI-TI for the Netflix dataset

Figure C.3.1.2.3-4: SI-TI for the EBU, SVT, 4ever and CableLabs dataset

Editor’s Note: To be added later: reference to the SI-TI scripts will be available (ETSI git)

#### C.3.1.2.4	SDR Sequence Selection

The first step consists in classifying the sequences in two categories: easy and challenging. This reflects the complexity of the sequences from an encoding perspective. Based on the R-D data, the QP points covering the application range [10-40] Mbps are identified. For the lower (10Mbps) and higher (40Mbps) boundaries, a linear interpolation is achieved between the two closest QP points to estimate the distortion at the boundaries: D_10 and D_40. If the distortion difference D_DIFF=(D_40-D_10) is less than or equal to a threshold Thr=1.5, then the sequence is classified as follows:

-	Low-dynamic: the PSNR dynamic on the application bitrate range is limited

-	High-dynamic: the PSNR dynamic on the application bitrate range is high

The results of this first classification step are reported in Table C.3.2.1.4-1.

Table C.3.2.1.4-1: SDR sequence classification based on R-D information in the [10-40] Mbps range, Thr=1.5

|  | Name | D_10 | D_40 | D_DIFF | Dynamic |
| --- | --- | --- | --- | --- | --- |
| JVET | Tango2 | 40.33 | 41.44 | 1.12 | Low |
|  | FoodMarket4 | 40.28 | 42.37 | 2.09 | High |
|  | Campfire | 35.06 | 37.29 | 2.23 | High |
|  | Catrobots1 | 38.36 | 39.85 | 1.50 | Low |
|  | Daylightroad2 | 37.42 | 38.61 | 1.19 | Low |
|  | Parkrunning3 | 31.17 | 33.65 | 2.48 | High |
| UVG | CityAlley | 43.37 | 43.49 | 0.12 | Low |
|  | FlowerFocus | 41.13 | 41.33 | 0.20 | Low |
|  | FlowerKids | 40.92 | 42.78 | 1.86 | High |
|  | FlowerPan | 39.60 | 39.85 | 0.25 | Low |
|  | RaceNight | 37.82 | 38.62 | 0.79 | Low |
|  | RiverBank | 36.62 | 38.84 | 2.22 | High |
|  | SunBath | 43.85 | 46.98 | 3.13 | High |
|  | Twilight | 42.92 | 43.24 | 0.33 | Low |
| Netflix | Netflix_Aerial | 38.62 | 41.00 | 2.37 | High |
|  | Netflix_DinnerScene | 42.85 | 43.08 | 0.23 | Low |
|  | Netflix_RollerCoaster | 42.61 | 44.73 | 2.11 | High |
|  | Netflix_DrivingPOV | 37.27 | 39.54 | 2.27 | High |
|  | Netflix_BarScene | 41.39 | 41.62 | 0.23 | Low |
|  | Netflix_ToddlerFountain | 33.56 | 36.01 | 2.45 | High |
|  | Netflix_Dancers | 41.96 | 42.14 | 0.18 | Low |
|  | Netflix_PierSeaside | 41.05 | 43.10 | 2.06 | High |
|  | Netflix_WindAndNature | 43.35 | 45.09 | 1.75 | High |
|  | Netflix_Boat | 35.27 | 38.64 | 3.37 | High |
|  | Netflix_FoodMarket | 37.62 | 38.95 | 1.32 | Low |
|  | Netflix_Tango | 40.31 | 41.27 | 0.97 | Low |
|  | Netflix_BoxingPractice | 40.97 | 42.14 | 1.17 | Low |
|  | Netflix_Narrator | 45.33 | 46.36 | 1.03 | Low |
|  | Netflix_TunnelFlag | 39.47 | 42.06 | 2.59 | High |
|  | Netflix_RitualDance | 40.25 | 43.46 | 3.21 | High |
|  | Netflix_FoodMarket2 | 39.11 | 41.27 | 2.16 | High |
|  | Netflix_SquareAndTimelapse | 39.43 | 42.15 | 2.72 | High |
| EBU | Rain_fruits | 40.04 | 41.69 | 1.65 | High |
|  | Lupo_candlelight | 39.81 | 40.01 | 0.20 | Low |
| SVT | CrowdRun | 31.41 | 33.94 | 2.53 | High |
|  | ParkJoy | 29.86 | 32.13 | 2.27 | High |
|  | DucksTakeOff | 29.31 | 30.75 | 1.44 | Low |
|  | IntoTree | 35.06 | 35.45 | 0.39 | Low |
|  | OldTownCross | 33.93 | 34.06 | 0.13 | Low |
| 4ever | 4EVER_Brest_Sedof | 28.66 | 31.42 | 2.76 | High |
|  | 4EVER_Paris_Montmartre | 30.33 | 33.19 | 2.86 | High |
| CableLabs | LiftingOff | 45.01 | 48.31 | 3.30 | High |
|  | Soccer | 43.31 | 44.89 | 1.58 | High |
|  | ElDorado | 39.41 | 44.29 | 4.88 | High |
|  | MomentsOfIntensity | 43.09 | 44.66 | 1.57 | High |

It is observed that 26 sequences are classified as "High" while 19 are "Low". In order to keep sequences that are relevant for a codec characterization, the low-dynamic sequences are removed. Then, the next step consists in removing sequences having some similarities, using SI-TI information computed in clause C.3.1.2.3. The sequences are represented below in the SI-TI plane with a k-means clustering to help the decision.

Figure C.3.1.2.4-1: SI-TI clustering for the Challenging sequences (10 clusters)

In order to remain representative in the sense of sources origin, format, and SI-TI values, the following approach was taken:

-	1 sequence (out of 6) from the C2 cluster (EBU: Rain-fruits)

-	1 sequence (out of 4) from the C3 cluster (CableLabs: Soccer)

-	2 sequences (out of 8) from the C4 cluster (Netfllix: ToddlerFountain, Boat)

-	1 sequence (out of 1)  from the C5 cluster (UVG: Riverbank)

-	1 sequence (out of 2) from the C6 cluster (Netflix: TunnelFlag)

-	1 sequence (out of 1) from the C8 cluster (SVT: ParkJoy)

-	1 sequence (out of 1) from the C10 cluster (4ever: Brest-Sedof)

The following eight sequences are selected:

-	1 From 4ever: Brest-Sedof (3840x2160p 60fps)

-	1 From EBU: RainFruits (3840x2160p 50fps)

-	1 From SVT: ParkJoy (3840x2160p 50fps)

-	1 From CableLabs: Soccer (3840x2160p 23.98fps)

-	3 From Netflix: TunnelFlag, Boat and Toddlerfountain (4096x2160p 59.94fps)

-	1 From UVG: RiverBank (3840x2160p 50fps)

### C.3.1.3	Selected Sequences

#### C.3.1.3.1	Brest-Sedof

The Brest-Sedof test sequence is tbd.

Figure C.3.1.3.1-1 shows a screenshot of the Brest-Sedof test sequence.

Figure C.3.1.3.1-1: Screenshot of Brest-Sedof test sequence

The Brest-Sedof test sequence properties are provided in Table C.3.1.3.1-1.

Table C.3.1.3.1-1 Brest-Sedof sequence properties

| Parameter | Value |
| --- | --- |
| Resolution | 3840 x 2160 |
| Scan | progressive |
| Frame Rate | 60 |
| Bit Depth | 10 |
| Length | 600 frames |
| YUV format | 4:2:0 |
| Colour components | ITU-T BT.2020 |
| Colour space | SDR |

The sequence can be accessed

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/Brest-Sedof/Brest-Sedof.json

The Brest-Sedof test sequence is made available under the following copyright disclaimer.

Copyright © …

#### C.3.1.3.2	Rain Fruits

The Rain Fruits test sequence is tbd.

Figure C.3.1.3.2-1 shows a screenshot of the Rain Fruits test sequence.

Figure C.3.1.3.2-1: Screenshot of Rain Fruits test sequence

The Rain Fruits test sequence properties are provided in Table C.3.1.3.2-1.

Table C.3.1.3.2-1 Rain Fruits sequence properties

| Parameter | Value |
| --- | --- |
| Resolution | 3840 x 2160 |
| Scan | progressive |
| Frame Rate | 50 |
| Bit Depth | 10 |
| Length | 600 frames |
| YUV format | 4:2:0 |
| Colour components | ITU-T BT.709 |
| Colour space | SDR |

The sequence can be accessed

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/Rain-Fruits/Rain-Fruits.json

The Rain Fruits test sequence is made available under the following copyright disclaimer.

Copyright © …

#### C.3.1.3.3	Park Joy

The Park Joy test sequence is tbd.

Figure C.3.1.3.3-1 shows a screenshot of the Park Joy test sequence.

Figure C.3.1.3.3-1: Screenshot of Park Joy test sequence

The Park Joy test sequence properties are provided in Table C.3.1.3.3-1.

Table C.3.1.3.3-1 Park Joy sequence properties

| Parameter | Value |
| --- | --- |
| Resolution | 3840 x 2160 |
| Scan | progressive |
| Frame Rate | 50 |
| Bit Depth | 10 |
| Length | 500 frames |
| YUV format | 4:2:0 |
| Colour components | ITU-T BT.709 |
| Colour space | SDR |

The sequence can be accessed

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/Park-Joy/Park-Joy.json

The Park Joy test sequence is made available under the following copyright disclaimer.

Copyright © …

#### C.3.1.3.4	Soccer

The Soccer test sequence is tbd.

Figure C.3.1.3.4-1 shows a screenshot of the Soccer test sequence.

Figure C.3.1.3.4-1: Screenshot of Brest-Sedof test sequence

The Soccer test sequence properties are provided in Table C.3.1.3.4-1.

Table C.3.1.3.4-1 Soccer sequence properties

| Parameter | Value |
| --- | --- |
| Resolution | 3840 x 2160 |
| Scan | progressive |
| Frame Rate | 24/1.001 |
| Bit Depth | 10 |
| Length | 408 frames |
| YUV format | 4:2:0 |
| Colour components | ITU-T BT.709 |
| Colour space | SDR |

The sequence can be accessed

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/Soccer/Soccer.json

The Soccer test sequence is made available under the following copyright disclaimer.

Copyright © …

#### C.3.1.3.5	Tunnel Flag

The Tunnel Flag test sequence is tbd.

Figure C.3.1.3.5-1 shows a screenshot of the Tunnel Flag test sequence.

Figure C.3.1.3.5-1: Screenshot of Tunnel Flag test sequence

The Tunnel Flag test sequence properties are provided in Table C.3.1.3.5-1.

Table C.3.1.3.5-1 Tunnel Flag sequence properties

| Parameter | Value |
| --- | --- |
| Resolution | 4096 x 2160 |
| Scan | progressive |
| Frame Rate | 60/1.001 |
| Bit Depth | 10 |
| Length | 600 frames |
| YUV format | 4:2:0 |
| Colour components | ITU-T BT.709 |
| Colour space | SDR |

The sequence can be accessed

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/Tunnel-Flag/Tunnel- Flag.json

The Tunnel Flag test sequence is made available under the following copyright disclaimer.

Copyright © …

#### C.3.1.3.6	Boat

The Boat test sequence is tbd.

Figure C.3.1.3.6-1 shows a screenshot of the Boat test sequence.

Figure C.3.1.3.6-1: Screenshot of Boat test sequence

The Boat test sequence properties are provided in Table C.3.1.3.6-1.

Table C.3.1.3.6-1 Boat sequence properties

| Parameter | Value |
| --- | --- |
| Resolution | 4096 x 2160 |
| Scan | progressive |
| Frame Rate | 60/1.001 |
| Bit Depth | 10 |
| Length | 300 frames |
| YUV format | 4:2:0 |
| Colour components | ITU-T BT.709 |
| Colour space | SDR |

The sequence can be accessed

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/Boat/boat.json

The Boat test sequence is made available under the following copyright disclaimer.

Copyright © …

#### C.3.1.3.7	Fountain

The Fountain test sequence is tbd.

Figure C.3.1.3.7-1 shows a screenshot of the Fountain test sequence.

Figure C.3.1.3.7-1: Screenshot of Fountain test sequence

The Fountain test sequence properties are provided in Table C.3.1.3.7-1.

Table C.3.1.3.7-1 Fountain sequence properties

| Parameter | Value |
| --- | --- |
| Resolution | 4096 x 2160 |
| Scan | progressive |
| Frame Rate | 60/1.001 |
| Bit Depth | 10 |
| Length | 1199 frames |
| YUV format | 4:2:0 |
| Colour components | ITU-T BT.709 |
| Colour space | SDR |

The sequence can be accessed

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/Fountain/Fountain.json

The Fountain test sequence is made available under the following copyright disclaimer.

Copyright © …

#### C.3.1.3.8	Riverbank

The Riverbank test sequence is tbd.

Figure C.3.1.3.8-1 shows a screenshot of the Riverbank test sequence.

Figure C.3.1.3.8-1: Screenshot of Riverbank test sequence

The Riverbank test sequence properties are provided in Table C.3.1.3.8-1.

Table C.3.1.3.8-1 Riverbank sequence properties

| Parameter | Value |
| --- | --- |
| Resolution | 3840 x 2160 |
| Scan | progressive |
| Frame Rate | 50 |
| Bit Depth | 10 |
| Length | 600 frames |
| YUV format | 4:2:0 |
| Colour components | ITU-T BT.709 |
| Colour space | SDR |

The sequence can be accessed

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/Riverbank/Riverbank.json

The Riverbank test sequence is made available under the following copyright disclaimer.

Copyright © …

## C.3.2 	HDR Category

### C.3.2.1	HDR Candidate Test Sequences

#### C.3.2.1.1	Introduction

In this Annex, a wide range of HDR test material is reported. The candidate sequences are extracted from test material used in video processing community. The following sections describe the test material in terms of format characteristics and copyright.

#### C.3.2.1.2	AOM Sequences

The AOM sequences are used for the purpose of AV1 and AV2 development. The sequences are available for download at https://media.xiph.org/video/av2/ and an overview is provided in the Table C.3.2.1.2-1. The following copyrights are applicable:

- Sparks : http://creativecommons.org/licenses/by-nc-nd/4.0/

-	Meridian : https://creativecommons.org/licenses/by-nc-nd/4.0/

-	Sol Levante : https://creativecommons.org/licenses/by-nc-nd/4.0/

-	Cosmos Laundromat : https://creativecommons.org/licenses/by/3.0/

-	Nocturne : https://creativecommons.org/licenses/by-nc-nd/4.0/

Table C.3.2.1.2-1: AOM test material description

| Name | Resolution | Frame Rate | Colour Gamut | TF | duration |
| --- | --- | --- | --- | --- | --- |
| sparks_aom_30-511 | 4096 x 2160 | 59.94 | P3-D65 | PQ | 481 |
| sparks_aom_2024-2455 | 4096 x 2160 | 59.94 | P3-D65 | PQ | 431 |
| sparks_aom_5363-5763 | 4096 x 2160 | 59.94 | P3-D65 | PQ | 400 |
| sparks_aom_5764-6024 | 4096 x 2160 | 59.94 | P3-D65 | PQ | 260 |
| sparks_aom_6026-6502 | 4096 x 2160 | 59.94 | P3-D65 | PQ | 476 |
| sparks_aom_8396-8941 | 4096 x 2160 | 59.94 | P3-D65 | PQ | 545 |
| sparks_aom_9774-10071 | 4096 x 2160 | 59.94 | P3-D65 | PQ | 297 |
| sparks_aom_11198-11570 | 4096 x 2160 | 59.94 | P3-D65 | PQ | 372 |
| meridian_aom_1782-2163 | 3840 x 2160 | 59.94 | P3-D65 | PQ | 381 |
| meridian_aom_11872-12263 | 3840 x 2160 | 59.94 | P3-D65 | PQ | 391 |
| meridian_aom_12264-12745 | 3840 x 2160 | 59.94 | P3-D65 | PQ | 481 |
| meridian_aom_15932-16309 | 3840 x 2160 | 59.94 | P3-D65 | PQ | 377 |
| meridian_aom_20988-21412 | 3840 x 2160 | 59.94 | P3-D65 | PQ | 424 |
| meridian_aom_22412-22738 | 3840 x 2160 | 59.94 | P3-D65 | PQ | 324 |
| meridian_aom_24058-24550 | 3840 x 2160 | 59.94 | P3-D65 | PQ | 492 |
| sol_levante_aom_289-453 | 3840 x 2160 | 24 | P3-D65 | PQ | 164 |
| sol_levante_aom_519-649 | 3840 x 2160 | 24 | P3-D65 | PQ | 130 |
| sol_levante_aom_2268-2412 | 3840 x 2160 | 24 | P3-D65 | PQ | 144 |
| sol_levante_aom_3282-3874 | 3840 x 2160 | 24 | P3-D65 | PQ | 592 |
| sol_levante_aom_4123-4545 | 3840 x 2160 | 24 | P3-D65 | PQ | 422 |
| cosmos_aom_1573-1749 | 3840 x 2160 | 24 | P3-D65 | PQ | 176 |
| cosmos_aom_8686-8826 | 3840 x 2160 | 24 | P3-D65 | PQ | 176 |
| cosmos_aom_9561-9789 | 3840 x 2160 | 24 | P3-D65 | PQ | 228 |
| cosmos_aom_11589-11752 | 3840 x 2160 | 24 | P3-D65 | PQ | 163 |
| cosmos_aom_12025-12075 | 3840 x 2160 | 24 | P3-D65 | PQ | 50 |
| cosmos_aom_12149-12330 | 3840 x 2160 | 24 | P3-D65 | PQ | 181 |
| cosmos_aom_12916-13078 | 3840 x 2160 | 24 | P3-D65 | PQ | 162 |
| cosmos_aom_13446-13649 | 3840 x 2160 | 24 | P3-D65 | PQ | 203 |
| nocturne_aom_2370-2539 | 3840 x 2160 | 60 | P3-D65 | PQ | 169 |
| nocturne_aom_8540-9009 | 3840 x 2160 | 60 | P3-D65 | PQ | 469 |
| nocturne_aom_9010-9349 | 3840 x 2160 | 60 | P3-D65 | PQ | 339 |
| nocturne_aom_17140-17709 | 3840 x 2160 | 60 | P3-D65 | PQ | 569 |
| nocturne_aom_18013-18315 | 3840 x 2160 | 60 | P3-D65 | PQ | 302 |
| nocturne_aom_23820-24322 | 3840 x 2160 | 60 | P3-D65 | PQ | 502 |
| nocturne_aom_27740-28109 | 3840 x 2160 | 60 | P3-D65 | PQ | 369 |
| nocturne_aom_32660-32799 | 3840 x 2160 | 60 | P3-D65 | PQ | 139 |

The clip cosmos_aom_12025-12075 being fifty frames duration, it is removed from the selection process and is not considered. In order to use these sequences for the selection process, a conversion into the appropriate format is conducted using FFmpeg and HDRTools software v0.21. The conversion according to Figure C.3.2.1.2-1 conducted:

Figure C.3.2.1.2-1: Processing pipeline for AOM sequence conversion

In the first step FFMPEG was used to decode JPEG 2000 sequences and store in uncompressed TIFF format. The following command line was used:

FFMPEG -i $INPUT.j2k -compression_algo raw -pix_fmt rgb48le $OUTPUT.tiff

HDRConvert was used to convert the sequences from P3-D65 RGB 12 bit to Rec. BT.2100 10 bit YCbCr 4:2:0. HDRConvert config file used for this process is attached to this contribution.

#### C.3.2.1.3	CableLabs Sequences

The CableLabs dataset is composed by one HDR sequence described in Table C.3.2.1.3-1. The sequence can be used under the CC BY-NC-ND 3.0 license restriction.

Table C.3.2.1.3-1: Cable Labs HDR test material description

| Name | Resolution | Frame rate | Colour Gamut | TF | duration |
| --- | --- | --- | --- | --- | --- |
| Life Untouched | 4096 x 2160 | 59.94 | P3-D65 | PQ | 5:20 |

For this sequence, a selection of some critical time intervals have been conducted based on experience on compression. Especially intervals with fast or complex motion, representative of TV contents have been selected. Static pictures, or pictures with artistic intent with few spatial details have been discarded. To remain in the scope of the scenario, the following sections have been selected as final candidates:

- Life Untouched 1: 0-8s

- Life Untouched 2: 8-15s

The extraction of subpart from original ProRes sequences is realized with FFmpeg release 4.3.1 using the following command line:

./ffmpeg.exe -i $IN.mov -vcodec rawvideo -ss $START_TIME -t $DURATION -pix_fmt yuv420p10le $OUT.yuv

Where $IN and $OUT are respectively the input and output sequences, $START_TIME and $DURATION respectively the start time and duration in hh:mm:ss format.

In order to map the color into a BT.2020 color container, HDRTools v2.1 has been used, using the same process as the one described in clause C.3.2.1.1. HDRConvert config file used for this process is attached to this contribution.

### C.3.2.2	Selected Test Sequences

#### C.3.2.2.1	Introduction

In order to decide which sequences should be used in this specification a similar process as the SDR one is used. The following process is conducted:

- Run informative encoding using x265 on the converted candidate sequences

- Run SI-TI metrics computation on the converted candidate sequences

- Run codeword histogram computation on the converted candidate sequences

- Select a diverse datasets based on R-D, SI-TI and codewords distribution.

#### C.3.2.2.2	x265 Encoding

To represent candidate sequences in the R-D plane, fixed QP encodings are carried out for QPs [12, 17, 22, 27, 32, 37, 42,5 1] using ffmpeg release 4.3.1, with the following command line:

./ffmpeg.exe -pix_fmt yuv420p10le -s $Wx$H -framerate $FR -i $IN -vcodec libx265 -qp $QP -preset fast -x265-params “psnr=1:ssim=1” $OUT.mp4

Where $W and $W are respectively the sequence width and height, $FR the sequence frame rate, $QP the selected quantization parameter. $IN and $OUT are respectively the input and output files.

From the obtained log, the overall bitrate and PSNR are extracted to be used for plotting. The encoding result are presented in

- Figure C.3.2.2.2-1 R-D curves for the AOM dataset (Nocturne clips),

- Figure C.3.2.2.2-2 R-D curves for the AOM dataset (Meridian clips),

- Figure C.3.2.2.2-3 R-D curves for the AOM dataset (Cosmos clips),

- Figure C.3.2.2.2-4 R-D curves for the AOM dataset (Sparks clips),

- Figure C.3.2.2.2-5 R-D curves for the AOM dataset (SolLevante clips), and

- Figure C.3.2.2.2-6 R-D curves for the Cable Labs dataset.

All sequences being represented in the targeted [10-40] Mbps range (red box).

Figure C.3.2.2.2-1: R-D curves for the AOM dataset (Nocturne clips)

Figure C.3.2.2.2-2: R-D curves for the AOM dataset (Meridian clips)

Figure C.3.2.2.2-3: R-D curves for the AOM dataset (Cosmos clips)

Figure C.3.2.2.2-4: R-D curves for the AOM dataset (Sparks clips)

Figure C.3.2.2.2-5: R-D curves for the AOM dataset (SolLevante clips)

TBD (cablelabs)

Figure C.3.2.2.2-6: R-D curves for the Cable Labs dataset

#### C.3.2.2.3	SI-TI Metrics

The analysis is based on spatial perceptual information (SI) and temporal perceptual information (TI) metrics calculated as defined in ITU-P.910 recommendation [40]. In order to reduce the bias introduced by scene-cuts in TI metric computation, the TI at scene-cut boundaries are removed from computation.

The TI metrics are presented in

- Figure C.3.2.2.3-1 SI-TI for the AOM dataset (Nocturne clips),

- Figure C.3.2.2.3-2 R SI-TI for the AOM dataset (Meridian clips),

- Figure C.3.2.2.3-3 SI-TI for the AOM dataset (Cosmos clips),

- Figure C.3.2.2.3-4 SI-TI for the AOM dataset (Sparks clips), and

- Figure C.3.2.2.3-5 SI-TI for the AOM dataset (SolLevante clips) and CableLabs dataset.

Figure C.3.2.2.3-1: SI-TI for the AOM dataset (Sparks clips)

Figure C.3.2.2.3-2: SI-TI for the AOM dataset (Meridian clips)

Figure C.3.2.2.3-3: SI-TI for the AOM dataset (Nocturne clips)

Figure C.3.2.2.3-4: SI-TI for the AOM dataset (Cosmos clips)

TBD (cablelabs)

Figure C.3.2.2.3-5: SI-TI for the AOM (SolLevante) and CableLabs dataset

#### C.3.2.2.4	Codewords distribution

For the HDR case, the distribution of codewords into the sequence is also taken into account in the decision process. The obtained histograms are provided in

- Figure C.3.2.2.4-1 Codewords distribution for the AOM dataset (Nocturne clips),

- Figure C.3.2.2.4-2 Codewords distribution for the AOM dataset (Meridian clips),

- Figure C.3.2.2.4-3 Codewords distribution for the AOM dataset (Cosmos clips),

- Figure C.3.2.2.4-4 Codewords distribution for the AOM dataset (Sparks clips),

- Figure C.3.2.2.4-5 Codewords distribution for the AOM dataset (SolLevante clips), and

- Figure C.3.2.2.4-6 Codewords distribution for the Cable Labs dataset.

Figure C.3.2.2.4-1: Codewords distribution for AOM dataset (Meridian clips)

Figure C.3.2.2.4-2: Codewords distribution for AOM dataset (Cosmos clips)

Figure C.3.2.2.4-3: Codewords distribution for AOM dataset (Sparks clips)

Figure C.3.2.2.4-4: Codewords distribution for AOM dataset (SolLevante clips)

Figure C.3.2.2.4-5: Codewords distribution for AOM dataset (Nocturne clips)

TBD (cablelabs)

Figure C.3.2.2.4-6: Codewords distribution for CableLabs dataset

#### C.3.2.2.5	HDR Sequence Selection

The first step consists in classifying the sequences in two categories: easy and challenging. This reflects the complexity of the sequences from an encoding perspective. Based on the R-D data, the QP points covering the application range [10-40] Mbps are identified. For the lower (10Mbps) and higher (40Mbps) boundaries, a linear interpolation is achieved between the two closest QP points to estimate the distortion at the boundaries: D_10 and D_40. If the distortion difference D_DIFF=(D_40-D_10) is less than or equal to a threshold Thr=3, then the sequence is classified as follows:

- Low-dynamic: the PSNR dynamic on the application bitrate range is limited

- High-dynamic: the PSNR dynamic on the application bitrate range is high

The results of this first classification step is reported in the Table C.3.2.2.5-1.

Table C.3.2.2.5-1: HDR sequence classification based on R-D information in [10-40] Mbps range, Thr=3

|  | Name | D_10 | D_40 | D_DIFF | Complexity |
| --- | --- | --- | --- | --- | --- |
| Sparks | sparks_aom_30-511 | 55.93 | 58.88 | 2.95 | Low |
|  | sparks_aom_2024-2455 | 38.95 | 43.28 | 4.33 | High |
|  | sparks_aom_5363-5763 | 40.24 | 44.16 | 3.92 | High |
|  | sparks_aom_5764-6024 | 33.45 | 37.29 | 3.84 | High |
|  | sparks_aom_6026-6502 | 34.54 | 38.69 | 4.16 | High |
|  | sparks_aom_8396-8941 | 43.35 | 44.46 | 1.10 | Low |
|  | sparks_aom_9774-10071 | 47.15 | 48.89 | 1.74 | Low |
|  | sparks_aom_11198-11570 | 42.45 | 44.36 | 1.91 | Low |
| Nocturne | nocturne_aom_2370-2539 | 49.81 | 52.14 | 2.33 | Low |
|  | nocturne_aom_8540-9009 | 48.50 | 51.39 | 2.89 | Low |
|  | nocturne_aom_9010-9349 | 48.50 | 51.39 | 2.89 | Low |
|  | nocturne_aom_17140-17709 | 50.83 | 53.05 | 2.23 | Low |
|  | nocturne_aom_18013-18315 | 42.57 | 44.37 | 1.80 | Low |
|  | nocturne_aom_23820-24322 | 50.84 | 55.62 | 4.78 | High |
|  | nocturne_aom_27740-28109 | 40.37 | 45.54 | 5.17 | High |
|  | nocturne_aom_32660-32799 | 45.04 | 50.25 | 5.22 | High |
| Meridian | meridian_aom_1782-2163 | 42.08 | 42.91 | 0.83 | Low |
|  | meridian_aom_11872-12263 | 41.81 | 42.44 | 0.62 | Low |
|  | meridian_aom_12264-12745 | 43.15 | 43.85 | 0.70 | Low |
|  | meridian_aom_15932-16309 | 41.81 | 42.44 | 0.62 | Low |
|  | meridian_aom_20988-21412 | 44.11 | 44.34 | 0.24 | Low |
|  | meridian_aom_22412-22738 | 40.43 | 42.03 | 1.60 | Low |
|  | meridian_aom_24058-24550 | 43.35 | 43.57 | 0.22 | Low |
| SolLevante | sol_levante_aom_289-453 | 30.57 | 38.02 | 7.45 | High |
|  | sol_levante_aom_519-649 | 24.23 | 26.81 | 2.58 | Low |
|  | sol_levante_aom_2268-2412 | 32.42 | 39.30 | 6.89 | High |
|  | sol_levante_aom_3282-3874 | 31.50 | 39.30 | 7.81 | High |
|  | sol_levante_aom_4123-4545 | 35.95 | 41.22 | 5.27 | High |
| Cosmos | cosmos_aom_1573-1749 | 43.32 | 47.37 | 4.04 | High |
|  | cosmos_aom_8686-8826 | 48.12 | 49.85 | 1.73 | Low |
|  | cosmos_aom_9561-9789 | 51.75 | 54.25 | 2.50 | Low |
|  | cosmos_aom_11589-11752 | 45.46 | 48.02 | 2.56 | Low |
|  | cosmos_aom_12149-12330 | 42.55 | 47.49 | 4.93 | High |
|  | cosmos_aom_12916-13078 | 49.76 | 52.29 | 2.53 | Low |
|  | cosmos_aom_13446-13649 | 49.84 | 52.09 | 2.25 | Low |
| CableLabs | LifeUntouched-1 | 45.26 | 49.89 | 4.63 | High |
|  | LifeUntouched-2 | 38.70 | 41.36 | 2.66 | Low |

It is observed that 14 sequences are classified as "High" while 23 are "Low". In order to keep sequences that are relevant for a codec characterization, the low-dynamic sequences are removed. To select a data set having different sources, LifeUntouched_1 is automatically selected since it is the only complex one in CableLabs set. Similarly, the sequence meridian_aom_22412-22738 from Meridian which is most challenging from an encoding perspective is selected.

Then, the next step consists in selecting a subset of remaining sequences in order to end up with a reasonable number of sequences, from diverse sources and having different characteristics. With the target of selecting the same number of sequences as SDR, the remaining set of sequences are clustered into the SI-TI plane, using k-mean clustering to identify sequences having same spatial and temporal features. Since two sequences have already been selected, and given that eight sequences are targeted, six clusters are identified in the figure below.

Figure C.3.2.2.5-1: SI-TI clustering for the challenging HDR sequences (6 clusters)

It is observed that clusters C3 and C4 only contain one sequence each, which are automatically selected since they present interesting codewords distribution and content. For other clusters, the codeword histograms are used to help deciding which sequence should be selected.

For C5, sparks_aom_2024-2455 is selected since it presents interesting codewords distribution compared to what is already included in the figure above.

For C1, sol_levante_aom_289-453 is rejected since it is similar to the cosmos sequence. In the two remaining sequences, nocturne_aom_32660-32799 is too short in duration while nocturne_aom_23820-24322 does not present particular interest due to a static long dark transition part in the middle. Thus, no sequences are selected from C1.

For C6, sol_levante sequences are removed since two computer-generated sequences are already selected. sparks_aom_5764-6024 is selected to make the dataset more diverse in terms of codeword distributions.

For C2, cosmos_aom_1573-1749 is removed since two computer-generated sequences are already selected. For the sake of having diverse sources, nocturne_aom_27740-28109 is selected since two sparks sequences are already selected.

Eventually, the selected sequences are summarized below:

-	LifeUntouched1 (TBD)

-	meridian_aom_22412-22738

-	sol_levante_aom_2268-2412

-	cosmos_12149-12330

-	sparks_aom_2024-2455

-	sparks_aom_5764-6024

-	nocturne_aom_27740-28109

### C.3.2.3	Selected Sequences

#### C.3.2.3.1	Life Untouched

The Life Untouched test sequence is tbd(LifeUntouched1).

Figure C.3.2.3.1-1 shows a screenshot of the Life Untouched test sequence.

Figure C.3.2.3.1-1: Screenshot of Life Untouched test sequence

The Life Untouched test sequence properties are provided in Table C.3.2.3.1-1.

Table C.3.2.3.1-1 Life Untouched sequence properties

| Parameter | Value |
| --- | --- |
| Resolution | 4096 x 2160 |
| Scan | progressive |
| Frame Rate | 60/1.001 |
| Bit Depth | 10 |
| Length | 480 |
| YUV format | 4:2:0 |
| Colour components | ITU-T BT.2020 |
| Colour space | HDR PQ |

The sequence can be accessed

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/Life-Untouched/Life-Untouched.json

The Life Untouched test sequence is made available under the following copyright disclaimer.

Copyright © …

#### C.3.2.3.2	Meridian

The Meridian test sequence is tbd(meridian_aom_22412-22738).

Figure C.3.2.3.2-1 shows a screenshot of the Meridian test sequence.

Figure C.3.2.3.2-1: Screenshot of Meridian test sequence

The Meridian test sequence properties are provided in Table C.3.2.3.2-1.

Table C.3.2.3.2-1 Meridian sequence properties

| Parameter | Value |
| --- | --- |
| Resolution | 3840 x 2160 |
| Scan | progressive |
| Frame Rate | 60/1.001 |
| Bit Depth | 10 |
| Length | 324 |
| YUV format | 4:2:0 |
| Colour components | ITU-T BT.2020 |
| Colour space | HDR PQ |

The sequence can be accessed

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/Meridian/Meridian.json

The Meridian test sequence is made available under the following copyright disclaimer.

Copyright © …

#### C.3.2.3.3	Sol Levante

The Sol Levante test sequence is tbd(sol_levante_aom_2268-2412).

Figure C.3.2.3.3-1 shows a screenshot of the Sol Levante test sequence.

Figure C.3.2.3.3-1: Screenshot of Sol Levante test sequence

The Sol Levante test sequence properties are provided in Table C.3.2.3.3-1.

Table C.3.2.3.3-1 Sol Levante sequence properties

| Parameter | Value |
| --- | --- |
| Resolution | 3840 x 2160 |
| Scan | progressive |
| Frame Rate | 24 |
| Bit Depth | 10 |
| Length | 144 |
| YUV format | 4:2:0 |
| Colour components | ITU-T BT.2020 |
| Colour space | HDR PQ |

The sequence can be accessed

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/Sol-Levante/Sol-Levante.json

The Sol Levante test sequence is made available under the following copyright disclaimer.

Copyright © …

#### C.3.2.3.4	Cosmos

The Cosmos test sequence is tbd(cosmos_12149-12330).

Figure C.3.2.3.4-1 shows a screenshot of the Cosmos test sequence.

Figure C.3.2.3.4-1: Screenshot of Cosmos test sequence

The Cosmos test sequence properties are provided in Table C.3.2.3.4-1.

Table C.3.2.3.4-1 Cosmos sequence properties

| Parameter | Value |
| --- | --- |
| Resolution | 3840 x 2160 |
| Scan | progressive |
| Frame Rate | 24 |
| Bit Depth | 10 |
| Length | 181 |
| YUV format | 4:2:0 |
| Colour components | ITU-T BT.2020 |
| Colour space | HDR PQ |

The sequence can be accessed

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/Cosmos/Cosmos.json

The Cosmos test sequence is made available under the following copyright disclaimer.

Copyright © …

#### C.3.2.3.5	Elevator

The Elevator test sequence is tbd(sparks_aom_2024-2455).

Figure C.3.2.3.5-1 shows a screenshot of the Elevator test sequence.

Figure C.3.2.3.5-1: Screenshot of Elevator test sequence

The Elevator test sequence properties are provided in Table C.3.2.3.5-1.

Table C.3.2.3.5-1 Elevator sequence properties

| Parameter | Value |
| --- | --- |
| Resolution | 4096 x 2160 |
| Scan | progressive |
| Frame Rate | 60/1.001 |
| Bit Depth | 10 |
| Length | 431 |
| YUV format | 4:2:0 |
| Colour components | ITU-T BT.2020 |
| Colour space | HDR PQ |

The sequence can be accessed

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/Elevator/Elevator.json

The Elevator test sequence is made available under the following copyright disclaimer.

Copyright © …

#### C.3.2.3.6	Sparks

The Sparks test sequence is tbd(sparks_aom_5764-6024).

Figure C.3.2.3.6-1 shows a screenshot of the Sparks test sequence.

Figure C.3.2.3.6-1: Screenshot of Sparks test sequence

The Sparks test sequence properties are provided in Table C.3.2.3.6-1.

Table C.3.2.3.6-1 Sparks sequence properties

| Parameter | Value |
| --- | --- |
| Resolution | 4096 x 2160 |
| Scan | progressive |
| Frame Rate | 60/1.001 |
| Bit Depth | 10 |
| Length | 260 |
| YUV format | 4:2:0 |
| Colour components | ITU-T BT.2020 |
| Colour space | HDR PQ |

The sequence can be accessed

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/Sparks/Sparks.json

The Sparks test sequence is made available under the following copyright disclaimer.

Copyright © …

#### C.3.2.3.7	Nocturne

The Nocturne test sequence is tbd(nocturne_aom_27740-28109).

Figure C.3.2.3.7-1 shows a screenshot of the Nocturne test sequence.

Figure C.3.2.3.7-1: Screenshot of Nocturne test sequence

The Nocturne test sequence properties are provided in Table C.3.2.3.7-1.

Table C.3.2.3.7-1 Nocturne sequence properties

| Parameter | Value |
| --- | --- |
| Resolution | 4096 x 2160 |
| Scan | progressive |
| Frame Rate | 60/1.001 |
| Bit Depth | 10 |
| Length | 369 |
| YUV format | 4:2:0 |
| Colour components | ITU-T BT.2020 |
| Colour space | HDR PQ |

The sequence can be accessed

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/Nocturne/Nocturne.json

The Nocturne test sequence is made available under the following copyright disclaimer.

Copyright © …

## C.3.3 	HFR Category

Tbd

# C.4		HD Test Sequences

## C.4.1 	Overview

tbd

## [C.4.2	CTA WAVE Tears of Steel

### C.4.2.1	Introduction

CTA WAVE has produced a derivate of Tears of Steel with dedicated overlays for testing purposes. The content has been subsampled and different frame rates are produced. Details can be found here: http://dash.akamaized.net/WAVE/index.html. Figure C.4.2.1-1 shows a screenshot of the Screenshot of CTA WAVE’s ToS test sequence.

Figure C.4.2.1-1: Screenshot of CTA WAVE’s ToS test sequence

### C.4.2.2	Sequence properties

The CTA WAVE’s ToS test sequence properties are provided in Table C.4.2.2-1.

Table C.4.2.2-1 CTA WAVE’s ToS test sequences

| Parameter | WAVE-ToS-01 | WAVE-ToS-02 | WAVE-ToS-03 |
| --- | --- | --- | --- |
| Resolution | 1920x1080 | 1600x900 | 1280x720 |
| Scan | Progressive | Progressive | Progressive |
| Frame Rate | 60 fps | 60 fps | 60 fps |
| Bit Depth | 8 | 8 | 8 |
| Length | 3600 frames (60s) | 3600 frames (60s) | 3600 frames (60s) |
| YUV format | YUV 4:2:0 | YUV 4:2:0 | YUV 4:2:0 |
| Color components | Y’CbCr | Y’CbCr | Y’CbCr |
| Colour space | ITU-R BT.709 | ITU-R BT.709 | ITU-R BT.709 |

The original version as well as subsampled ones are available

-	WAVE-ToS-01: http://dash.akamaized.net/WAVE/Mezzanine/tos_L1_1920x1080@60_60.mp4

-	WAVE-ToS-02: http://dash.akamaized.net/WAVE/Mezzanine/tos_K1_1600x900@60_60.mp4

-	WAVE-ToS-03: http://dash.akamaized.net/WAVE/Mezzanine/tos_J1_1280x720@60_60.mp4

### C.4.2.3	Copyright and license information

The Teers of Steel video sequence is made available under the following copyright disclaimer.

The proceedings and results of the Mango Open Movie project are being licensed under the Creative Commons Attribution 3.0 license. This material is a result of the Mango Open Movie project, who owns the copyright.  The Mango Open Movie project (licensor) has licensed it under the Creative Commons Attribution 3.0 license (https://creativecommons.org/licenses/by/3.0/). In short, this means you can freely reuse and distribute this content, also commercially, for as long you provide a proper attribution.

The attribution is: (CC) Blender Foundation | mango.blender.org

## C.4.3	CTA WAVE Croatia

### C.4.3.1	Introduction

CTA WAVE has produced a derivate of Tears of Steel with dedicated overlays for testing purposes. The content has been subsampled and different frame rates are produced. Details can be found here: http://dash.akamaized.net/WAVE/index.html. Figure C.4.3.1-1 shows a screenshot of the Screenshot of CTA WAVE’s Croatia test sequence.

Figure C.4.3.1-1: Screenshot of CTA WAVE’s Croatia test sequence

### C.4.3.2	Sequence properties

The CTA WAVE’s Croatia test sequence properties are provided in Table C.4.3.2-1.

Table C.4.3.2-1 CTA WAVE’s Croatia test sequences

| Parameter | WAVE-CRO-01 | WAVE-CRO-02 | WAVE-CRO-03 |
| --- | --- | --- | --- |
| Resolution | 1920x1080 | 1600x900 | 1280x720 |
| Scan | Progressive | Progressive | Progressive |
| Frame Rate | 50 fps | 50 fps | 50 fps |
| Bit Depth | 10 | 10 | 10 |
| Length | 3600 frames (60s) | 3600 frames (60s) | 3600 frames (60s) |
| YUV format | YUV 4:2:0 | YUV 4:2:0 | YUV 4:2:0 |
| Color components | Y’CbCr | Y’CbCr | Y’CbCr |
| Colour space | ITU-R BT.2100 PQ | ITU-R BT.2100 PQ | ITU-R BT.2100 PQ |

The original version as well as subsampled ones are available

-	WAVE-ToS-01: http://dash.akamaized.net/WAVE/Mezzanine/croatia_L1_1920x1080@50_60.mp4

-	WAVE-ToS-02: http://dash.akamaized.net/WAVE/Mezzanine/croatia_K1_1600x900@50_60.mp4

-	WAVE-ToS-03: http://dash.akamaized.net/WAVE/Mezzanine/croatia_J3_1280x720@50_60.mp4

### C.4.3.3	Copyright and license information

The Croatia video sequence is made available under the following copyright disclaimer.

This material has been recorded by the EBU, who owns the copyright. The EBU (licensor) has licensed it under the Creative Commons Attribution 4.0 license (https://creativecommons.org/licenses/by/4.0/). In short, this means you can freely reuse and distribute this content, also commercially, for as long you provide a proper attribution.

# C.5		Messaging Sequences

[

## C.5.1 	Overview

https://photos.app.goo.gl/6QrmTTMoizVtxAoQA

## C.5.2	Powder

### C.5.2.1	Introduction

A donated sequence with outdoor filming.

Figure C.5.2.1-1: Screenshot of Powder sequence

### C.5.2.2	Sequence properties

The Powder test sequence properties are provided in Table C.5.2.2-1.

Table C.5.2.2-1 Powder test sequence

| Parameter | POWDER -01 |
| --- | --- |
| Resolution |  | 1920x1080 |  |
| Scan |  | Progressive |  |
| Frame Rate |  | 30 fps |  |
| Bit Depth |  | 8 |  |
| Length |  | 3600 frames (60s) |  |
| YUV format |  | YUV 4:2:0 |  |
| Color components |  | Y’CbCr |  |
| Colour space |  | ITU-R BT.709 |  |

### C.5.2.3	Copyright and license information

The Powder video sequence is made available under the following copyright disclaimer.

This material has been recorded by the Thomas Stockhammer, who owns the copyright. Thomas Stockhammer (licensor) has licensed it under the Creative Commons Attribution 4.0 license (https://creativecommons.org/licenses/by/4.0/). In short, this means you can freely reuse and distribute this content, also commercially, for as long you provide a proper attribution.

# C.6		Screen Content Test Sequences

## C.6.1 	Overview

Screen content test sequences have been generated for the purpose of this study with the objective to illustrate commonly encountered situations during videoconferences, particularly when sharing presentation slides. Some critical configurations are part of this test set such as moving text at different speeds and various transitions between the slides.

## C.6.2	Moving Text 2

### C.6.2.1	Introduction

The sequence MovingText2 is composed of several lines of text horizontally scrolling at different speeds over a gradation of background colours. The sequence has been generated in 4K resolution at 60 frames per second with an authoring tool and exported into Apple ProRes 422 before being converted into YUV4:2:0. The figure C.6.2.1-1 is a screenshot of the MovingText2 test sequence.

Figure C.6.2.1-1: Screenshot of MovingText2 test sequence

### C.6.2.2	Source sequence properties

The MovingText2 test sequence properties are provided in Table C.6.2.2-1.

Table C.6.2.2-1 Moving Text 2 test sequences

| Parameter | MT2-01 | MT2-02 | MT2-03 | MT2-04 |
| --- | --- | --- | --- | --- |
| Resolution | 3840 x 2160 | 3840 x 2160 | 1920 x1080 | 1920 x1080 |
| Scan | Progressive | Progressive | Progressive | Progressive |
| Frame Rate | 60 fps | 60 fps | 60 fps | 60 fps |
| Bit Depth | 10 | 8 | 10 | 8 |
| Length | 600 frames (10s) | 600 frames (10s) | 600 frames (10s) | 600 frames (10s) |
| YUV format | YUV 4:2:0 | YUV 4:2:0 | YUV 4:2:0 | YUV 4:2:0 |
| Color components | Y’CbCr | Y’CbCr | Y’CbCr | Y’CbCr |
| Colour space | ITU-R BT.709 | ITU-R BT.709 | ITU-R BT.709 | ITU-R BT.709 |

The original version as well as subsampled ones are available here:

- FullHD 8bit: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/MovingText2-FullHD-8bit/

- FullHD 10bit: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/MovingText2-FullHD-10bit/

- 4K 8bit: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/MovingText2-4K-8bit/

- 4K 10bit: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/MovingText2-4K-10bit/

### C.6.2.3	Copyright information

The MovingText2 video sequence has been produced for the purpose of this study on the characterization of video codecs in 3GPP.

## C.6.3	Text Mix Transitions

### C.6.3.1	Introduction

The sequence TextMixTransitions is composed of 2 slides containing fixed text at different sizes. Those 2 slides appear twice each during 2.5s. 1s long transitions are inserted in between (fade, slide, turning page). The sequence has been generated in 4K resolution at 60 frames per second with an authoring tool and exported into Apple ProRes 422 before being converted into YUV4:2:0. The figure C.6.3.1-1 is a screenshot of the TextMixTransitions test sequence.

Figure C.6.3.1-1: Screenshot of TextMixTransitions test sequence

### C.6.3.2	Source sequence properties

The TextMixTransitions test sequence properties are provided in Table C.6.3.2-1.

Table C.6.3.2-1 TextMixTransitions test sequences

| Parameter | TMT-01 | TMT-02 | TMT-03 | TMT-04 |
| --- | --- | --- | --- | --- |
| Resolution | 3840 x 2160 | 3840 x 2160 | 1920 x1080 | 1920 x1080 |
| Scan | Progressive | Progressive | Progressive | Progressive |
| Frame Rate | 60 fps | 60 fps | 60 fps | 60 fps |
| Bit Depth | 10 | 8 | 10 | 8 |
| Length | 600 frames (10s) | 600 frames (10s) | 600 frames (10s) | 600 frames (10s) |
| YUV format | YUV 4:2:0 | YUV 4:2:0 | YUV 4:2:0 | YUV 4:2:0 |
| Color components | Y’CbCr | Y’CbCr | Y’CbCr | Y’CbCr |
| Colour space | ITU-R BT.709 | ITU-R BT.709 | ITU-R BT.709 | ITU-R BT.709 |

The original version as well as subsampled ones are available here:

- FullHD 8bit: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/TextMixTransitions-FullHD-8bit/

- FullHD 10bit: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/TextMixTransitions-FullHD-10bit/

- 4K 8bit: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/TextMixTransitions-4K-8bit/

- 4K 10bit: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/TextMixTransitions-4K-10bit/

### C.6.3.3	Copyright information

The TextMixTransitions video sequence has been produced for the purpose of this study on the characterization of video codecs in 3GPP.

## C.6.4	Graphics Mix Simple

### C.6.4.1	Introduction

The sequence GraphicsMixSimple is composed of 3 slides containing different types of graphics. Those 3 slides appear once each during 3.33s. Transitions are simple scene cut in between. The sequence has been generated in 4K resolution at 60 frames per second with an authoring tool and exported into Apple ProRes 422 before being converted into YUV4:2:0. The figure C.6.4.1-1 is a screenshot of each of the 3 scenes contained in the GraphicsMixSimple test sequence.

Figure C.6.4.1-1: Screenshot of the 3 scenes in GraphicsMixSimple test sequence

### C.6.4.2	Source sequence properties

The GraphicsMixSimple test sequence properties are provided in Table C.6.4.2-1.

Table C.6.4.2-1 GraphicsMixSimple test sequences

| Parameter | GMS-01 | GMS-02 | GMS-03 | GMX-04 |
| --- | --- | --- | --- | --- |
| Resolution | 3840 x 2160 | 3840 x 2160 | 1920 x1080 | 1920 x1080 |
| Scan | Progressive | Progressive | Progressive | Progressive |
| Frame Rate | 60 fps | 60 fps | 60 fps | 60 fps |
| Bit Depth | 10 | 8 | 10 | 8 |
| Length | 600 frames (10s) | 600 frames (10s) | 600 frames (10s) | 600 frames (10s) |
| YUV format | YUV 4:2:0 | YUV 4:2:0 | YUV 4:2:0 | YUV 4:2:0 |
| Color components | Y’CbCr | Y’CbCr | Y’CbCr | Y’CbCr |
| Colour space | ITU-R BT.709 | ITU-R BT.709 | ITU-R BT.709 | ITU-R BT.709 |

The original version as well as subsampled ones are available here:

- FullHD 8bit: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/GraphicsMixSimple-FullHD-8bit/

- FullHD 10bit: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/GraphicsMixSimple-FullHD-10bit/

- 4K 8bit: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/GraphicsMixSimple-4K-8bit/

- 4K 10bit: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/GraphicsMixSimple-4K-10bit/

### C.6.4.3	Copyright information

The GraphicsMixSimple video sequence has been produced for the purpose of this study on the characterization of video codecs in 3GPP.

## C.6.5	Graphics Mix Transition

### C.6.5.1	Introduction

The sequence GraphicsMixTransition is the same sequence as GraphicsMixSimple described in clause C.6.4, except the fact that 1s long transitions have been inserted composed of 3 slides containing different types of graphics. The 3 slides appear once each during 3.33s. Transitions are fading and page turning in between. The sequence has been generated in 4K resolution at 60 frames per second with an authoring tool and exported into Apple ProRes 422 before being converted into YUV4:2:0. The figure C.6.5.1-1 is a screenshot of each of the 3 scenes contained in the GraphicsMixTransition test sequence.

Figure C.6.5.1-1: Screenshot of the 2 transitions in GraphicsMixTransitions test sequence

### C.6.5.2	Source sequence properties

The GraphicsMixTransitions test sequence properties are provided in Table C.6.5.2-1.

Table C.6.5.2-1 GraphicsMixTransitions test sequences

| Parameter | GMT-01 | GMT-01 | GMT-03 | GMT-04 |
| --- | --- | --- | --- | --- |
| Resolution | 3840 x 2160 | 3840 x 2160 | 1920 x1080 | 1920 x1080 |
| Scan | Progressive | Progressive | Progressive | Progressive |
| Frame Rate | 60 fps | 60 fps | 60 fps | 60 fps |
| Bit Depth | 10 | 8 | 10 | 8 |
| Length | 600 frames (10s) | 600 frames (10s) | 600 frames (10s) | 600 frames (10s) |
| YUV format | YUV 4:2:0 | YUV 4:2:0 | YUV 4:2:0 | YUV 4:2:0 |
| Color components | Y’CbCr | Y’CbCr | Y’CbCr | Y’CbCr |
| Colour space | ITU-R BT.709 | ITU-R BT.709 | ITU-R BT.709 | ITU-R BT.709 |

The original version as well as subsampled ones are available here:

-	FullHD 8bit: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/GraphicsMixTransitions-FullHD-8bit/

-	FullHD 10bit: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/GraphicsMixTransitions-FullHD-10bit/

-	4K 8bit: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/GraphicsMixTransitions-4K-8bit/

-	4K 10bit: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/GraphicsMixTransitions-4K-10bit/

### C.6.5.3	Copyright information

The GraphicsMixTransitions video sequence has been produced for the purpose of this study on the characterization of video codecs in 3GPP.

Annex D: Reference Configurations

# D.1	Introduction

The reference configurations are attached to this document and also provided here: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Anchors/CFG/.

# D.2		H.264/AVC JM19.0 Reference Configurations

## D.2.1 	JM-01: Low-latency

Add details

# D.3		H.265/HEVC HM16.22 Reference Configurations

## D.3.1 	HM-01: Low-latency without Intra

https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Anchors/CFG/hm-01.cfg

## D.3.2 	HM-02: Low-latency with regular Intra

https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Anchors/CFG/hm-02.cfg

# D.4		H.265/HEVC SCM-8.8 Reference Configurations

## D.4.1 	SCC-01: Low-latency without Intra

https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Anchors/CFG/scc-01.cfg

## D.4.2 	SCC-02: Low-latency with regular Intra

https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Anchors/CFG/scc-02.cfg

Annex E Data Management and Hosting

# E.1	Introduction

All reference sequences and all anchors are hosted on a publicly accessible web-server here:

https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo

A frontend is provided here:

https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/index.html.

Editor’s Note: this frontend still needs to be implemented.

In order to address reference sequences with different raw formats and to maintain consistency, a json file has been added to describe the properties of the file.

# E.2	Reference Sequences

## E.2.1 	Hosting

All reference sequences are hosted at

https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences.

Below this folder, for each reference sequence a dedicated folder is created. The folder includes at least the following information:

-	JSON file for raw video according to schema in clause C.1.2.

In addition, the folder may include the following information

- 	the raw video sequence, if hosted on the server,

-	a preview file,

-	a thumbnail image.

## E.2.2 	Uploading

For uploading a new reference sequence, please create a new issue here

- https://github.com/haudiobe/5G-Video-Content/issues/new?assignees=&labels=request&template=reference_sequence.md&title=

Please specify all information that needs to be added to the JSON file for raw video sequences as defined in clause C.1.2.

## E.2.3 	Downloading

Sequences can be downloaded from the above folders. Licensing terms must be obeyed. Downloaded files should be MD5 verified.

On Windows

Open Command Prompt.

Open your downloads folder by typing cd Downloads. ...

Type certutil -hashfile followed by the file name and then MD5.

Check that the value returned matches the value the MD5 file you downloaded from the json.

On Mac

Open Terminal.

Type md5 and hit the SPACE button.

Drag the file you have downloaded into the Terminal Window. ...

Hit Enter.

You should now see the MD5 Checksum so you can compare it to the string you have been given in json.

On Linux

Open Terminal.

Type md5 and add file name – hit enter

Verify the json provided MD5.

# E.3	Anchors

## E.3.1 	Hosting

All anchors are hosted at:

https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Anchors.

Below this folder, a hierarchy is created

-	Scenario

-	Codec

-	Anchor Tuple

-	Anchor

Below this folder, for each anchor a dedicated folder is created.

The folder includes at least the following information:

-	JSON file for binary video according to schema in clause B.3.2.

-	binary video bitstream

The basic structure is as follows

Editor’s Note: needs update for final test sets

Anchors

- CFG/                              	22-Feb-2021 11:38      -

- Scenario-3/                       	01-Mar-2021 12:41      -

- Scenario-5/     				01-Mar-2021 17:09

Scenario

- Anchor definitions http://dash.akamaized.net/WAVE/3GPP/5GVideo/Anchors/Scenario-3/anchors.csv

- Metrics/                          26-Feb-2021 08:02

  - Metrics are summarized for each anchor

  - http://dash.akamaized.net/WAVE/3GPP/5GVideo/Anchors/Scenario-3/Metrics/s3-a01-265.csv

- S3-A01-265/ 			26-Feb-2021 14:49

  - S3-A01-265-22.json                26-Feb-2021 14:49     1k

  - S3-A01-265-27.json                26-Feb-2021 14:49     1k

  - S3-A01-265-32.json                26-Feb-2021 14:49     1k

  - S3-A01-265-37.json                26-Feb-2021 14:49     1k

  - S3-A01-265-42.json                26-Feb-2021 14:49     1k

  - S3-A01-265_22.bin                 26-Feb-2021 14:49  13.1M

  - S3-A01-265_27.bin                 26-Feb-2021 14:49   6.9M

  - S3-A01-265_32.bin                 26-Feb-2021 14:49   3.6M

  - S3-A01-265_37.bin                 26-Feb-2021 14:49   2.0M

  - S3-A01-265_42.bin                 26-Feb-2021 14:49   1.0M

- For each anchor the above directory is available

## E.3.2 	Uploading

For uploading anchors, please create a new issue here

- https://github.com/haudiobe/5G-Video-Content/issues/new?assignees=&labels=request&template=anchor_tuple.md&title=

Please specify all required information that needs to be added to the JSON file for encoded video sequences as defined in clause C.1.2.

## E.3.3 	Downloading

Anchors can be downloaded for free from the above folders. Licensing terms must be obeyed. Downloaded files should be MD5 verified.

# E.4	Tests

## E.4.1 	Hosting

All tests are hosted at:

https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Tests/

The basic structure is as follows

Editor’s Note: needs update for final test sets

- VTM/                              01-Mar-2021 19:17

  - CFG/                              01-Mar-2021 18:28      -

  - Scenario-3/                       01-Mar-2021 18:40      -

    - tests.csv                         01-Mar-2021 18:28     3k

    - Metrics/                          01-Mar-2021 18:29

      - S3-T01-VTM.csv                    01-Mar-2021 18:29     1k

    - S3-T01-VTM/                       01-Mar-2021 18:30      -

      - S3-T01-VTM-22.json                01-Mar-2021 18:29     1k

      - S3-T01-VTM-27.json                01-Mar-2021 18:29     1k

      - S3-T01-VTM-32.json                01-Mar-2021 18:29     1k

      - S3-T01-VTM-37.json                01-Mar-2021 18:29     1k

      - S3-T01-VTM-42.json                01-Mar-2021 18:29     1k

      - S3-T01-VTM_22.bin                 01-Mar-2021 18:29   9.1M

      - S3-T01-VTM_27.bin                 01-Mar-2021 18:29   4.8M

      - S3-T01-VTM_32.bin                 01-Mar-2021 18:29   2.6M

      - S3-T01-VTM_37.bin                 01-Mar-2021 18:30   1.4M

      - S3-T01-VTM_42.bin                 01-Mar-2021 18:30   765k

    - S3-T02-VTM/                       01-Mar-2021 18:30      -

  - Scenario-5/                       01-Mar-2021 19:17      -

# E.5	Verification

tbd

Annex F: Software Package

# F.1	Introduction

Editor’s Note: needs update for final scripting details

# F.2	Readme

## F.2.1	Quickstart

Given an anchor definition, the tool performs encoding, reconstruction and runs additional metrics computation for all variants.

1.	add the samples/references/yuv420_1280x720_8bit_rec709.yuv sequence

2.	set the environment variable: HM_ENCODER=/path/to/HM/bin/TAppEncoderStatic

3.	generate the sample HM anchor

./cmd.py ./samples/anchors/sample_hm.json encode decode`

## F.2.2	Using docker

A sample Dockerfile is provided to build an image containing all the dependencies: HM, JM, and VTM

git clone https://github.com/haudiobe/5GVideo.git

cd 5GVideo

docker build -t anchortools -f ./docker/Dockerfile .

# add the missing `./samples/references/yuv420_1280x720_8bit_rec709.yuv` sequence

docker run --mount type=bind,source=./samples,target=/samples -it anchortools cmd.py /samples/anchors/sample_hm.json

the --mount option mounts the directory source path, and makes it available as target path in the running container.

## F.2.3	Usage

### F.2.3.1	General

./cmd.py ./anchor.json [encode] [decode] [metrics]

•	the tool assumes you have reference encoders compiled, with environment variables pointing to the executables, see below.

### F.2.3.2	encode

./cmd.py ./anchor.json encode decode

runs the reference encoder to generate both bitstream, and reconstructed sequence for all the anchors, with the reconstructed chroma format as specified in the encoder config.

./cmd.py ./anchor.json encode

runs the reference encoder to generate bitstream only.

### F.2.3.3		decode

./cmd.py ./anchor.json decode

runs the reference decoder to reconstruct the bitstream, with the output chroma format matching the bitstream.

### F.2.3.4		metrics

./cmd.py ./anchor.json metrics

generates some metrics for each variant defined in the anchor

the metrics options uses third party tools, see below.

## F.2.4	Reference encoders

the following environment variables are needed depending on the encoder/decoder you want to use:

JM

JM_ENCODER=/path/to/JM/bin/lencod_static

JM_DECODER=/path/to/JM/bin/ldecod_static

HM

HM_ENCODER=/path/to/HM/bin/TAppEncoderStatic

HM_DECODER=/path/to/bin/TAppDecoderStatic

VTM

VTM_ENCODER=/path/to/bin/EncoderAppStatic

VTM_DECODER=/path/to/bin/DecoderAppStatic

Adding a custom encoder

implement the ReferenceEncoder interface (encoder_id, encode_variant, decode_variant) and decorate your class (@register_encoder)

## F.2.5	Anchor definition

### F.2.5.1	example anchor.json :

{

"description": "human readable description, use case, settings overview ...",

"reference": "path/to/reference/sample.yuv",

"encoder": "HM",

"encoder_cfg": "path/to/anchor/encoder_cfg.cfg",

"variants": {

"variant_id0": {

"OptionKey": "OptionValue0",

"OptionKey2": "{ANCHOR_DIR}/sub0.cfg",

[...]

},

"variant_id1": {

"OptionKey": "OptionValue1",

"OptionKey2": "{ANCHOR_DIR}/sub1.cfg",

[...]

},

[...]

}

}

NOTE:

- if test_sequence or encoder_cfg is a relative path, it is interpreted as relative to anchor.json.

- when used in the variant options {ANCHOR_DIR} is expanded to the anchor's encoder config directory: path/to/anchor.

### F.2.5.2	Variant options to encoder CLI args mapping

HM, VTM

{

"encoder_cfg": "/encoder.cfg",

"variants": {

"variant_id0": {

"-k": "v",

"--Key": "value"

}

}

}

maps to -c /encoder.cfg -k v --Key=value

JM

{

"encoder_cfg": "/encoder.cfg",

"variants": {

"variant_id0": {

"-f": "file.cfg",

"Key": "value"

}

}

}

maps to -d /encoder.cfg -f file.cfg -p Key=value

### F.2.5.3	Output

the above configuration would generate the following :

# variant bitstream

path/to/anchor/encoder_cfg.variant_id0.bit

# encoder log (VTM has additional .opl file)

path/to/anchor/encoder_cfg.variant_id0.enc.log

# reconstructed variant

path/to/anchor/encoder_cfg.variant_id0.yuv

# variant metrics

path/to/anchor/encoder_cfg.variant_id0.csv

[...]

path/to/anchor/encoder_cfg.variant_id1.bit

path/to/anchor/encoder_cfg.variant_id1.yuv

path/to/anchor/encoder_cfg.variant_id1.enc.log

path/to/anchor/encoder_cfg.variant_id1.csv

[...]

# averaged metrics, one variant per row

path/to/anchor/encoder_cfg.csv

## F.2.6	Raw video sequence description

YUV sequences are currently described through a sidecar file.

eg. for the above path/to/reference/sample.yuv, add the following path/to/reference/sample.json

{

"width": 1280,

"height": 720,

"chroma_format": "yuv",

"chroma_subsampling": "420",

"bitdepth": 8,

"fps": 30,

"color_space": "rec709",

"transfer": null,

"framecount": 30

}

## F.2.7	Current limitations

•	fps is converted to integer

•	only planar YUV reference sequences are supported

•	the transfer and color_space properties are currently ignored, however color space conversions can be configured through the encoder.cfg file and through variant options for each encoder

•	metrics computation assumes reference sequence and reconstructed sequences share the same chroma format, frame packing and bitdepth

## F.2.8	Dependencies [metrics]

The open-source gpac application is needed for metrics computation.

Detailed build instruction please refer to : https://github.com/gpac/gpac/wiki/Build-Introduction

the path to the gpac executable can be configured through environment variable, eg. :

GPAC_APP=/path/to/bin/gpac

Annex G: Attachments

# G.1	Introduction

The attachments to this document are provided in 26955-Attachments.zip.

Editor’s Note: needs update for final scripting details

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-04 | SA4#108 | S4-200661 |  |  |  | Initial Version | 0.0.1 |
| 2020-04 | SA4#108 | S4-200666 |  |  |  | Version agreed at SA4#108-e | 0.1.0 |
| 2020-05 | SA4#109 | S4-200891 |  |  |  | Version agreed at SA4#109-e | 0.2.0 |
| 2020-08 | SA4#110 | S4-201243 |  |  |  | Version agreed at SA4#110-e | 0.3.0 |
| 2020-11 | SA4#111 | S4-201515 |  |  |  | Version agreed at SA4#111-e | 0.4.0 |
| 2021-02 | SA4#112 | S4-210207 |  |  |  | Version agreed at SA4#112-e | 0.5.0 |
| 2021-03 | SA4#112 | S4aV200662 |  |  |  | Version agreed to be sent to SA plenary for information | 0.6.0 |