| 3GPP TS 28.567 V0.1.0 (2025-02) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Management aspects of closed control loops (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Concepts and overview	7

4.1	Closed Control Loops	7

4.2	Functional steps of a closed control loop	7

4.3	Characteristic information of a CCL	8

4.3.1	 Overview	8

4.3.1	CCL Goals and targets	8

4.3.2	CCL actions and actions plans	8

4.3.3	CCL scopes	8

4.4	Closed control loops purposes and uses	9

5	Management capabilities	10

5.1	Dynamic composition of CCLs - DynCCL	10

5.1.1	Description	10

5.1.2	 Use cases	10

5.1.3	Requirements	11

5.2	 Historical CCL Capability - HISCCL	11

5.2.1	 Description	11

5.2.2	Use Cases	12

5.2.2	Requirements	12

5.3	 CCL Performance Monitoring - CCLPERF	12

5.3.1	 Description	12

5.3.2	Use Cases	12

5.3.2	 Requirements	13

5.4	Closed Control Loops for fault management - <XY>	13

5.4.1	 Description	13

5.4.2	Use Cases	13

5.4.3	Requirements	14

6	Model	15

7	Procedures	16

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.535: "Management and orchestration; Management services for communication service assurance; Requirements".

[3]	3GPP TS 28.536: "Management and orchestration; Management services for communication service assurance; Stage 2 and stage 3".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

# 4	Concepts and overview

## 4.1	Closed Control Loops

A Closed Control Loop (CCL) is a type of control mechanism that monitors and regulates a set of managed entities with the objective of achieving a specific goal. A CCL can be logically decomposed into several stages or steps, each providing a specific functionality and where the steps work together to achieve the stated goal. Any two CCLs with the same functionality may have the functionality supported in different count of steps implementing the functionality and similarly, any two CCLs with the same functionality and same count of steps, the respective steps may not have the same functionality.

A control loop is a building block for management of networks and services. The basic principle of any control loop is to adjust the value of an observed variable (expressed as for example an attribute) to control/influence the value of a desired goal (expressed as for example an attribute) for a controlled entity, such as a managed entity or managed function. The producers of the measurements or observations services, analysis services and  control service, are all required to fully realize and use a control loop.

A Closed Control Loop (CCL) is a control loop which operates without any intervention from a human operator or any other management entity other than possibly the initial configuration of the measurement producer and configuration of the control loop. In a closed control loop the input to the control loop provided by human operator or other management entity may include the goal or policies. Besides the provisioning needed to realize the goal, the output of the closed control loop may also include closed control loop status to a human operator or other management entity.

Examples of well-known Closed Loop types are OODA loop, composed of 4 stages/steps (Observe, Orient, Decide, Act) and MAPE, also composed of 4 stages/steps (Monitor, Analyse, Plan, Execute).

- b)

Figure 4.1-1: Open control loop entities versus Closed control loop entities (see TS28.535[2])

## 4.2	Functional steps of a closed control loop

A closed control loop may manage any managed entity, e.g. a network resource or a communication service as described in [2] and [3]. Generally, the control loop consists of the steps Monitoring/data collection, Analysis, Decision and Execution. The adjustment of the resources of the managed entity used is completed by the continuous iteration of the steps in a management control loop.

Figure 4.1.2-1: Steps of a Control Loop
a) the four functional steps; and
b) 2 steps combined into a single management function

The "Monitoring/data collection" step is responsible for collecting and pre-processing data from managed entities or from external sources.

The "Analysis" step derives insights from the available data obtained in the monitoring/data collection step. The insights provide answers to the questions, for example What is likely to happen, what has happened and why.

The "Decision" step is responsible for deriving workflows from insights provided by the analysis step. It decides which reactive, proactive or predictive actions should be taken in consideration of insights obtained in the analysis step.

The execution step manages the activation of commands on the controlled resources or entities. The decision step should decide which actions are required, but not necessarily how they should be taken in the managed entities. So, the translation from actions to commands is a responsibility of the execution step.

## 4.3	Characteristic information of a CCL

### 4.3.1		Overview

A CCL is associated with a set of Characteristic information that describes its properties, behaviours and impact. This information includes CCL Goals, CCL Triggers, CCL actions and action plans as well as CCL scopes.

### 4.3.1	CCL Goals and targets

The CCL is the set of outcomes that need to be achieved or realized by the CCL. Each expected outcome is called a CCL target. A CCL may have one more CCL goals each containing a set of CCL targets.

### 4.3.2	CCL actions and actions plans

A CCL action is a configuration change that a CCL can perform over a managed entity such as configuring an attribute of  managed object. A CCL may decide to perform several actions which can be combined in a single CCL action plan.

### 4.3.3	CCL scopes

The scope is the set of managed objects, their properties and network outcomes that are associated with the CCL for measurement, configuration and impact. The scopes for the different CCLs can be managed by the MnS consumer, i.e. they can be defined on to the CCL or revised by the MnS consumer. A CCL may have four scopes - the measurement scope, target (impact) scope, control scope and impact scope, defined as follows:

- measurement scope: the measurement scope is where related measurements are collected

- control scope: control scope is the scope to which the CCL's actions are desired to be applied, e.g.,  the set of network functions and attributes that are the planned candidates to be modified by the CCL. The control scope is also called the action-space as it describes the set of candidate actions that the CCL can (is configured to be able to) execute.

- target scope: which relates to purpose of the CCL

- desired impact scope: the scope to which the CCL's actions are desired to have influence, e.g., it is both the network functions and attributes as well network outcomes like coverage areas that are planned to be influenced by the configuration’s actions of the CCL.

- impact scope: impact scope is the scope to which the CCL's actions have influence, e.g., it is both the network functions and attributes as well network outcomes like coverage areas that are influenced by the configurations actions of the CCL. This is different from the measurement scope, i.e. the scope where the CCLs measure and control scope, i.e. the scope where they act.

The impact scope may be known and bounded or unbounded and thus unknown - see figure 4.3.5.4-1. The bounded scope indicates that the area known by the CCL is the scope where its actions will impact. The unbounded impact-scope is the full network scope where the CCL’s action will have impact, but the CCL does have information that its action will have that impact to that scope.

Figure 4.3.5.4-1: Exemplification of known/bounded vs. unknown/unbounded impact scope: CCL A takes action in cell A expecting impact in cells A, B, C and D. if the impact is strictly in cells A, B, C and D, then the impact scope is known and bounded. However, if the impact scope includes cells E and F, then for the CCL, the true impact scope is unknown and thus unbounded.

## 4.4	Closed control loops purposes and uses

CCLs automate the management of network resources thereby taking control away from operators. TS28.535[2], TS28.536[3] describe the use of CCLs for service assurance. Additionally, CCLs may be used for several other use cases including among others use of CCLs for problem analysis and for fault management.

In each case, the CCL can be viewed as an entity to be managed, management capabilities related to the CCL will be exposed by the MnS producer that is associated with the CCL to enable the MnS consumer to manage the CCL. E.g., the characteristics and behaviours of the CCLs need to be directed by operators as consumers of CCL-related management services.  Moreover, the MnS also exposes capabilities for coordinating the CCL's activities. Example capabilities include capability for providing information on conflict resolution and feedback on monitoring the impact of the CCL's actions on other closed control loops or management functions.

The 3GPP management system should provide capabilities that enable a consumer to:

-	Manage the execution of CCLs. E.g. to request for and be notified about the instantiation of CCLs. For instance, if the consumer wants to request for instantiation of an Energy saving CCL for 10,000 cells.

-	Compose or request for and be notified about the composition of a CCL from a set of specific components (such as analytics services or SON functions).

-	Manage a closed loop composed from multiple components.

# 5	Management capabilities

## 5.1	Dynamic composition of CCLs - DynCCL

### 5.1.1	Description

CCLs may be dynamically realized. There are two aspects to dynamically realization of CCLs - dynamic instantiation of a CCL from an existing template and dynamically composing the CCL from discrete components based e.g. on the provided goals.

### 5.1.2		Use cases

5.1.2.1	Composing a CCL from discrete components - Disc

A CCL may be composed from steps provided by different management functions or management services. i.e. the CCLs is assembled on demand by MnS consumers, using capabilities offered by the Management system, e.g. from independent management functions. The CCLs components, as well as the communication and interoperation between components, are based the different 3GPP management services. Accordingly, the MnS consumer should be able to identify and indicate the MnFs or MnS producers that should be used to compose a CCL. Moreover, the MnS consumer may indicate towards the MnS producer the request to compose the CL of a particular type (e.g. for optimizing energy efficiency) without requiring to state the specific components that should be used.

Two approaches are possible:

-	Composition from management Functions: Different management functions may be used to realize the different steps of a closed loop, for example, an MDA function may realize the analytics step of the CCL while another management function may realize the decision step of the CCL.

Figure 5.1.2.1-1: Management functions as steps of a closed control loop

-	Composition from management services: Different management services may be used to realize the different steps of a closed loop, i.e. the management service provides the output expected from a specific step.

EXAMPLE:	A capability of the MDA MnS realizes an analytics step of the CCL while another capability may realize a specific data collection step of the CCL.

a)	b)

Figure 5.1.2.2-1: Management services used as implementations of CCL steps:
a) MDA MnS and PM job the respective implementations of the analysis and data collection steps; and b) MDA MnS as the implementation of the decision step

The MnS consumer should be enabled to control the composition of such a CCL. The MnS consumer could request for and be notified about the composition of a CCL from a set of specific components (i.e. specific management functions or management services). The request could indicate components with specific given capabilities (such as analytics services with specific analytics types) which should be combined to achieve the closed loop. Moreover, the request could be for composition of a CCL required to achieve a specific set of desired outcomes or goals.

### 5.1.3	Requirements

| Requirement label | Description | Related use case(s)/ Motivation |
| --- | --- | --- |
| REQ-DynCCL-Disc-01 | The CCL MnS Producer should support a capability enabling the MnS consumer to request for a CCL (instance) to be composed from a set of management function types or instances or management services | UC-DynCCL-Disc Clause 5.1.2.1 |
| REQ-DynCCL-Disc-01 | The MnS producer for CCL management should support a capability enabling the MnS consumer to request that a CCL of a specific type or fulfilling a stated goal should be composed from a set of management function types or instances or services | UC-DynCCL-Disc Clause 5.1.2.1 |
| REQ-DynCCL-Disc-01 | The MnS producer for CCL management should support a capability enabling the MnS consumer to provide conditions under which a CCL can be dynamically composed or instantiated triggered to execute | UC-DynCCL-Disc Clause 5.1.2.1 |
| REQ-DynCCL-Disc-01 | The MnS producer for CCL management should support a capability enabling the MnS consumer to be notified when a CCL is dynamically composed or instantiated or triggered to execute | UC-DynCCL-Disc Clause 5.1.2.1 |

## 5.2		Historical CCL Capability - HISCCL

### 5.2.1		Description

TBD

### 5.2.2	Use Cases

5.2.2.1	CCL creation based on Historical CCL data capability - HISDAT

This use case describes the need of maintaining information about the CCLs that existed in the past. Those CCLs are called Historical CCLs.

In an automation environment, before a consumer request to create a CCL it would like to know the data related with Historical CCLs that were available with the producer. This information will enable consumer to request for an optimal CCL. The information about historical CCL may include, scope of the CCL, configured goals/targets, controlled entity, etc.

Further, Historical CCL information serves as a valuable data source for predictive analytics within the CCL system executed as Analytics step. It enables the system to move from a reactive mode, where it responds to current issues, to a proactive mode, where it anticipates and prevents problems based on historical trends and patterns. This proactive approach enhances network reliability, minimizes downtime, and improves the overall efficiency of network operations.

The Historical CCL information may be used by the management system to setup or initialize a CCL. The Historical CCL information provides the profiles of a CCL for CCL at different hierarchies. For example, CCLs that do not do coordination which are at a lower hierarchy L and CCLs responsible for coordination (as coordination entities) which are at a higher hierarchy H. For a new CCL at a lower hierarchy, the management system obtains the profiles of the several CCLs at different hierarchies and correlates the information of the new CCL (e.g. its goal information) against the profiles of the CCLs at the different hierarchies. Based on this, the management system computes the complete profile of the new CCL (including e.g. its measurement and control scope) which is then configured onto the new CCL.

### 5.2.2	Requirements

| Requirement label | Description | Related use case(s)/ Motivation |
| --- | --- | --- |
| REQ-HISCCL-HISDAT-01 | The 3GPP management system should enable authorized MnS consumer to request for information (e.g. CCL identification, configured goals/targets and the related status, scope of the CCL, conflict information) related with Historical CCL. | UC-HISCCL-HISDAT Clause 5.2.2.1 |
| REQ-HISCCL-HISDAT-02 | The 3GPP management system shall have the capability to configure the profile of a CCL based on the historical CCL information that describes the profile of other CCLs at different hierarchies. | UC-HISCCL-HISDAT Clause 5.2.2.1 |

## 5.3		CCL Performance Monitoring - CCLPERF

### 5.3.1		Description

TBD

### 5.3.2	Use Cases

5.3.2.1	Performance Evaluation of a Closed Control Loop - EVAL

The advanced monitoring functionalities of a CCL can provide real-time insights into the performance and outcomes of a CCL. The monitoring activity for a Closed Control Loop may result in further actions that happen in the operation phase, e.g., evaluate and update, in order to change the closed control loop settings and improve its performance. So, there is a need to evaluate the performance of a Closed Control Loop itself. Such metrics are important to understand and change a CCL's behaviour and to improve its performance to pursue the assigned goal(s).

For example, certain performance aspects of a CCL can be very crucial to know in order to evaluate and decide upon a CCL's performance, such as the number of breached goals, time taken to meet a breached goal, number of conflicts occurred by a CCL etc. With the knowledge of such performance aspects of an existing CCL, a MnS consumer can more effectively update or create a new CCL.

An operator can also compare different CCLs based on these performances and choose the best one for its network deployment.

### 5.3.2		Requirements

| Requirement label | Description | Related use case(s)/ Motivation |
| --- | --- | --- |
| REQ-CCLPERF-EVAL-1 | The 3GPP management system should be able to obtain a CCL's performance with respect to the total number of occurrences of a goal breach. | UC-CCLPERF-EVAL Clause 5.3.2.1 |
| REQ-CCLPERF-EVAL-2 | The 3GPP management system should be able to obtain a CCL's performance with respect to the time taken by CCL to meet a breached goal. | UC-CCLPERF-EVAL Clause 5.3.2.1 |
| REQ-CCLPERF-EVAL-3 | The 3GPP management system should be able to obtain a CCL's performance with respect to the total number of conflicts occurred by a CCL | UC-CCLPERF-EVAL Clause 5.3.2.1 |

## 5.4	Closed Control Loops for fault management - <XY>

### 5.4.1		Description

TBD

### 5.4.2	Use Cases

5.4.2.1	Closed Control Loops for fault management - CCLFM

This use case describes a scenario in which an MnS consumer may request a CCL for fault management. The consumer may request to identify the root cause of the fault and take actions to mitigate and/or resolve the root cause for a given list of alarms. Furthermore, the request may include policies and actions specified by an MnS consumer in order to mitigate and/or resolve the root causes for the given alarms.

Based on the request, a CCL may take action to further enhance the correlation of alarms, for example, correlation of alarms with change in PM/KPIs and/or fault supervision events to find solutions to mitigate and/or resolve the identified root causes.  In addition, a fault management CCL may clear the alarms that otherwise have to be manually cleared by the MnS consumer, which are defined as ADMC Alarms  in TS 28.111.

The MnS producer reports the result of fault management. The report may include information regarding the status of each alarm, including any identified root cause and correlation information, which may indicate the successful mitigation and/or resolving of root causes for any given alarm.

### 5.4.3	Requirements

| Requirement label | Description | Related use case(s)/ Motivation |
| --- | --- | --- |
| REQ-CCLPPS-CCLFM-01 | The 3GPP management system shall have the capability to allow MnS consumer to request a closed control loop for fault management | UC-XY-CCLFM |
| REQ-CCLPPS-CCLFM-02 | The 3GPP management system shall have the capability to allow MnS consumer to get a report from the closed control loop regarding the status of fault management | UC-XY-CCLFM |

# 6	Model

# 7	Procedures

Annex A (normative): 
Solution sets

Annex B (informative):
PlantUML source code for NRM class diagrams

Annex C:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-02 | - | n/a | - | - | - | Initial skeleton | 0.0.0 |
| 2025-02 | SA5#159 | S5-251107 S5-251137 S5-251123 S5-251124 S5-250944 S5-250945 S5-250943 | pCR | - | - | 28.567-001.docx pCR TS28.567 TS clause structure.docx pCR TS28.567 Concepts and overview.docx pCR TS28.567 Dynamic composition of CCLs.docx Rel-19 pCR 28.567 CCL creation based on historical data UC and Requirements.docx Rel-19 pCR TS 28.567 Add use case and requirements for Performance monitoring of Closed control loop.docx Rel19 pCR TS28.567 Add use case and requirements for CCL for Fault Mgmt.docx | 0.1.0 |