| 3GPP TS 36.133 V19.1.0 (2025-06) |  |
| --- | --- |
| Technical Specification¦Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	103

1	Scope	105

2	References	105

3	Definitions, symbols and abbreviations	107

3.1	Definitions	107

3.2	Symbols	109

3.3	Abbreviations	110

3.4	Test tolerances	113

3.5	Additional notation	113

3.5.1	Groups of bands	113

3.5.1A	Groups of bands for satellite access	116

3.6	General	117

3.6.1	Applicability of requirements in this specification version	117

3.6.1.1	Applicability of requirements for UE capable of network-based CRS interference mitigation	123

3.6.1.2	Applicability of requirements with CRS muting for category M1 UE capable of CRS muting	125

3.6.1.3	Applicability of requirements with CRS muting for category M2 UE capable of CRS muting	126

3.6.2	Applicability of requirements for EN-DC operation	127

3.6.3	Applicability of requirements for NE-DC operation	128

3.6.4	Applicability of requirements for NGEN-DC operation	129

4	E-UTRAN RRC_IDLE state mobility	129

4.1	Cell Selection	129

4.2	Cell Re-selection	129

4.2.1	Introduction	129

4.2.2	Requirements	129

4.2.2.1	Measurement and evaluation of serving cell	131

4.2.2.2	Void	132

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	132

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	133

4.2.2.5	Measurements of inter-RAT cells	136

4.2.2.5.1	Measurements of UTRAN FDD cells	136

4.2.2.5.2	Measurements of UTRAN TDD cells	137

4.2.2.5.3	Measurements of GSM cells	139

4.2.2.5.4	Measurements of HRPD cells	140

4.2.2.5.5	Measurements of cdma2000 1X	141

4.2.2.5.6	Measurements of NR cells	142

4.2.2.5.7	Measurements of NR cells subject to CCA	145

4.2.2.5.8	Measurements of NR cells for RedCap	147

4.2.2.6	Evaluation of cell re-selection criteria	149

4.2.2.7	Maximum interruption in paging reception	149

4.2.2.8	void	150

4.2.2.9	UE measurement capability	150

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	151

4.2.2.10	Reselection to CSG cells	151

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	151

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	152

4.2.2.11	Void	153

4.2.2.12	Void	153

4.2.2.13	Void	153

4.2.2.14	UE measurement capability for RedCap	153

4.3	Minimization of Drive Tests (MDT)	154

4.3.1	Introduction	154

4.3.2	Measurements	154

4.3.2.1	Requirements	154

4.3.3	Relative Time Stamp Accuracy	154

4.3.3.1	Requirements	154

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	155

4.3.4.1	 Requirements	155

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	155

4.3.5.1	Requirements for timeSinceFailure	155

4.4	MBSFN Measurements	155

4.4.1	Introduction	155

4.4.2	MBSFN RSRP measurements	155

4.4.3	MBSFN RSRQ measurements	156

4.4.4	MCH BLER measurements	156

4.5	Proximity-based Services	156

4.5.1	Introduction	156

4.5.2	Requirements	156

4.5.2.1	Interruptions with ProSe Direct Discovery	156

4.5.2.2	Interruptions with ProSe Direct Communication	156

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	157

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	157

4.6	Cell Selection and Re-selection Requirements for UE category NB1	158

4.6.1	Cell Selection	158

4.6.2	Cell Re-selection	158

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	158

4.6.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	159

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	160

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	161

4.6.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	163

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	164

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	165

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	166

4.6.2.7	Maximum interruption in paging reception in normal coverage	168

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	168

4.6.2.8	UE measurement capability	168

4.6.2.9	WUS receptions for NB1	168

4.6.3	Requirements for transmission using preconfigured uplink resources for UE category NB1	169

4.6.3.1	Introduction	169

4.6.3.2	Requirements on UE synchronization for transmission using PUR	169

4.6.3.3	Requirements on TA validation for transmission using PUR	169

4.6A	Cell Selection and Re-selection Requirements for UE category NB-IoT for Satellite Access	170

4.6A.1	Cell Selection	170

4.6A.2	Cell Re-selection for UE category NB-IoT for Satellite Access	170

4.6A.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	171

4.6A.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	172

4.6A.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	173

4.6A.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	175

4.6A.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	177

4.6A.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	178

4.6A.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	180

4.6A.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	182

4.6A.2.7	Maximum interruption in paging reception in normal coverage	184

4.6A.2.7A	Maximum interruption in paging reception in enhanced coverage	184

4.6A.2.8	UE measurement capability	185

4.6A.2.9	WUS receptions for NB1	185

4.6A.3	Requirements for transmission using preconfigured uplink resources for UE category NB-IoT for Satellite Access	186

4.6A.3.1	Introduction	186

4.6A.3.2	Requirements on UE synchronization for transmission using PUR	186

4.6A.3.3	Requirements on TA validation for transmission using PUR	186

4.7	Cell Selection and Re-selection Requirements for UE category M1	186

4.7.1	Cell Selection	186

4.7.2	Cell Re-selection	186

4.7.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	186

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	186

4.7.2.1.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in normal coverage	187

4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	188

4.7.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	190

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	192

4.7.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	192

4.7.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	193

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	193

4.7.2.2.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in enhaned coverage	194

4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	195

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	198

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	199

4.7.2.2.5	Maximum interruption in paging reception for Category M1 UEs in enhanced coverage	200

4.7.2.3	WUS receptions for UE category M1	200

4.7.3	Channel quality report for UE Category M1 in idle mode	200

4.7.4	Requirements for transmission using preconfigured uplink resources for UE category M1	201

4.7.4.1	Introduction	201

4.7.4.2	Requirements on UE synchronization for transmission using PUR	201

4.7.4.3	Requirements on TA validation for transmission using PUR	201

4.7A	Cell Selection and Re-selection Requirements for UE category M1 for Satellite Access	202

4.7A.1	Cell Selection	202

4.7A.2	Cell Re-selection for UE category M1 for Satellite Access	202

4.7A.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	203

4.7A.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	203

4.7A.2.1.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in normal coverage	205

4.7A.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	206

4.7A.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	209

4.7A.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	211

4.7A.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	211

4.7A.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	212

4.7A.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	212

4.7A.2.2.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in enhaned coverage	213

4.7A.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	215

4.7A.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	217

4.7A.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	220

4.7A.2.2.5	Maximum interruption in paging reception for Category M1 UEs in enhanced coverage	220

4.7A.2.3	WUS receptions for UE category M1	221

4.7A.3	Channel quality report for UE Category M1 in idle mode for Satellite Access	221

4.7A.4	Requirements for transmission using preconfigured uplink resources for UE category M1 for Satellite Access	222

4.7A.4.1	Introduction	222

4.7A.4.2	Requirements on UE synchronization for transmission using PUR	222

4.7A.4.3	Requirements on TA validation for transmission using PUR	222

4.8	Idle State Positioning Measurement Requirements for UE category NB1	222

4.8.1	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for normal coverage	222

4.8.1.1	RSTD Measurement Reporting Delay	224

4.8.2	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	224

4.8.2.1	RSTD Measurement Reporting Delay	225

4.8.3	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for normal coverage	226

4.8.3.1	RSTD Measurement Reporting Delay	227

4.8.4	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	228

4.8.4.1	RSTD Measurement Reporting Delay	229

4.8.5	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	230

4.8.5.1	Measurement Reporting Delay	231

4.8.6	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	231

4.8.6.1	Measurement Reporting Delay	232

4.8.7	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	233

4.8.7.1	Measurement Reporting Delay	234

4.8.8	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	234

4.8.8.1	Measurement Reporting Delay	235

4.9	Idle Mode CA Measurement	236

4.9.1	Introduction	236

4.9.2	Requirement	236

4.9.2.1	Detected cell requirement during state transition and Idle mode	236

4.9.2.2	Measurements of inter-frequency CA candidate cells	237

4.9.2.3	Measurements on serving cell	237

4A	E-UTRAN RRC_INACTIVE state mobility	238

4A.1	Cell Re-selection	238

4A.1.1	Introduction	239

4A.1.2	Requirements	239

4A.1.2.1	UE measurement capability	239

4A.1.2.2	Measurement and evaluation of serving cell	239

4A.1.2.3	Measurements of intra-frequency E-UTRAN cells	239

4A.1.2.4	Measurements of inter-frequency E-UTRAN cells	239

4A.1.2.5	Evaluation of cell re-selection criteria	239

4A.1.2.6	Maximum interruption in paging reception	239

4A.1.2.7	Measurements of inter-RAT NR cells	239

4A.1.2.8	UE measurement capability for RedCap	239

4A.1.2.9	Measurements of inter-RAT NR cells for RedCap	239

4A.2	Requirements for UE Category M1	240

4A.2.1	Introduction	240

4A.2.2	Cell Selection	240

4A.2.3	Cell Reselection	240

4A.2.3.1	Cell Re-selection requirements for UE category M1 in normal coverage	240

4A.2.3.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	240

4A.2.3.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	241

4A.2.3.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	241

4A.2.3.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	241

4A.2.3.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	241

4A.2.4	Channel quality report for UE Category M1 in idle mode	243

5	E-UTRAN RRC_CONNECTED state mobility	243

5.1	E-UTRAN Handover	244

5.1.1	Introduction	244

5.1.2	Requirements	244

5.1.2.1	E-UTRAN FDD – FDD	244

5.1.2.1.1	Handover delay	244

5.1.2.1.2	Interruption time	244

5.1.2.2	E-UTRAN FDD – TDD	245

5.1.2.2.1	(Void)	246

5.1.2.2.2	(Void)	246

5.1.2.3	E-UTRAN TDD – FDD	246

5.1.2.3.1	(Void)	246

5.1.2.3.2	(Void)	246

5.1.2.4	E-UTRAN TDD – TDD	246

5.1.2.4.1	Handover delay	246

5.1.2.4.2	Interruption time	246

5.1.2.5	E-UTRAN HD–FDD	248

5.1.2.5.1	Handover delay	248

5.1.2.5.2	Interruption time	248

5.1.2.6	E-UTRAN FDD – FDD conditional handover	249

5.1.2.6.1	Handover delay	249

5.1.2.6.2	Measurement time	250

5.1.2.6.3	Preparation time	250

5.1.2.6.4	Interruption time	250

5.1.2.7	E-UTRAN FDD – TDD conditional handover	250

5.1.2.8	E-UTRAN TDD – FDD conditional handover	251

5.1.2.9	E-UTRAN TDD – TDD conditional handover	251

5.2	Void	251

5.3	Handover to other RATs	251

5.3.1	E-UTRAN - UTRAN FDD Handover	251

5.3.1.1	Introduction	251

5.3.1.1.1	Handover delay	251

5.3.1.1.2	Interruption time	251

5.3.2	E-UTRAN - UTRAN TDD Handover	252

5.3.2.1	Introduction	252

5.3.2.2	Requirements	252

5.3.2.2.1	Handover delay	252

5.3.2.2.2	Interruption time	252

5.3.3	E-UTRAN - GSM Handover	253

5.3.3.1	Introduction	253

5.3.3.2	Requirements	253

5.3.3.2.1	Handover delay	253

5.3.3.2.2	Interruption time	253

5.3.4	E-UTRAN - NR FR1 Handover	253

5.3.4.1	Introduction	253

5.3.4.2	Handover delay	254

5.3.4.3	Interruption time	254

5.3.4A	E-UTRAN - NR FR1 Handover to target cell using CCA	254

5.3.4A.1	Introduction	254

5.3.4A.2	Handover delay	255

5.3.4A.3	Interruption time	255

5.3.4B	E-UTRAN - NR FR1 Handover for RedCap	256

5.3.4B.1	Introduction	256

5.3.4B.2	Requirements	256

5.3.5	E-UTRAN - NR FR2 Handover	256

5.3.5.1	Introduction	256

5.3.5.2	Handover delay	256

5.3.5.3	Interruption time	256

5.4	Handover to Non-3GPP RATs	257

5.4.1	E-UTRAN – HRPD Handover	257

5.4.1.1	Introduction	257

5.4.1.1.1	Handover delay	257

5.4.1.1.2	Interruption time	257

5.4.2	E-UTRAN – cdma2000 1X Handover	258

5.4.2.1	Introduction	258

5.4.2.1.1	Handover delay	258

5.4.2.1.2	Interruption time	258

5.5	E-UTRAN Handover for Cat-M1 UEs	258

5.5.1	Introduction	258

5.5.2	Requirements in CEModeA	259

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	259

5.5.2.1.1	Handover delay	259

5.5.2.1.2	Interruption time	259

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	259

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	260

5.5.2.3.1	Void	260

5.5.2.3.2	Void	260

5.5.3	Requirements in CEModeB	260

5.5.3.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	260

5.5.3.1.1	Handover delay	260

5.5.3.1.2	Interruption time	260

5.5.3.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	261

5.5.3.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	261

5.5A	E-UTRAN Handover for Cat-M1 UEs for Satellite Access	261

5.5A.1	Introduction	261

5.5A.2	Requirements in CEModeA	261

5.5A.2.1	E-UTRAN FDD – FDD HO for Cat-M1 FDD UEs	261

5.5A.2.1.1	Handover delay	261

5.5A.2.1.2	Interruption time	261

5.5A.2.2	E-UTRAN FDD – FDD HO for Cat-M1 HD – FDD UEs	262

5.5A.2.3	E-UTRAN FDD – FDD conditional HO for Cat-M1 FDD UEs	262

5.5A.2.3.1	Handover delay	262

5.5A.2.3.2	Measurement time	263

5.5A.2.3.3	Preparation time	264

5.5A.2.3.4	Interruption time	264

5.5A.2.4	E-UTRAN FDD – FDD conditional HO for Cat-M1 HD – FDD UEs	265

5.5A.3	Requirements in CEModeB	265

5.5A.3.1	E-UTRAN FDD – FDD HO for Cat-M1 FDD UEs	265

5.5A.3.1.1	Handover delay	265

5.5A.3.1.2	Interruption time	265

5.5A.3.2	E-UTRAN FDD – FDD HO for Cat-M1 HD – FDD UEs	266

5.5A.3.3	E-UTRAN FDD – FDD conditional HO for Cat-M1 FDD UEs	266

5.5A.3.3.1	Handover delay	266

5.5A.3.3.2	Measurement time	267

5.5A.3.3.3	Preparation time	268

5.5A.3.3.4	Interruption time	268

5.5A.3.4	E-UTRAN FDD – FDD conditional HO for Cat-M1 HD – FDD UEs	269

5.6	Void	269

5.7	E-UTRAN DAPS Handover	269

5.7.1	Introduction	269

5.7.2	Requirements	270

5.7.2.1	E-UTRAN FDD – FDD	270

5.7.2.1.1	DAPS Handover delay	270

5.7.2.1.2	Interruption time	270

5.7.2.2	E-UTRAN FDD – TDD	271

5.7.2.3	E-UTRAN TDD – FDD	271

5.7.2.4	E-UTRAN TDD – TDD	271

5.8	EN-DC Handover with PSCell	271

5.8.1	Introduction	271

5.8.1.1	Handover with PSCell Interruption time	271

5.8.1.2	Handover with PSCell - NR PSCell Change Delay requirements	272

5.9	EN-DC Handover with PSCell using CCA	273

5.9.1	Introduction	273

5.9.1.1	Handover with PSCell – E-UTRA HO Interruption time	273

5.9.1.2	Handover with PSCell - NR PSCell Change Delay requirements	273

6	RRC Connection Mobility Control	274

6.1	RRC Re-establishment	274

6.1.1	Introduction	274

6.1.2	Requirements	274

6.1.2.1	UE Re-establishment delay requirement	275

6.2	Random Access	275

6.2.1	Introduction	275

6.2.2	Requirements	275

6.2.2.1	Contention based random access	275

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	275

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	276

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	276

6.2.2.1.4	Void	276

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	276

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	276

6.2.2.2	Non-Contention based random access	276

6.2.2.2.1	Correct behaviour when receiving Random Access Response	276

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	276

6.2.3	Requirements for Cat-M1 UEs	276

6.2.3A	Random Access Requirements for Cat-M1 UEs for Satellite Access	277

6.3	RRC Connection Release with Redirection	277

6.3.1	Introduction	277

6.3.2	Requirements	277

6.3.2.1	RRC connection release with redirection to UTRAN FDD	277

6.3.2.2	RRC connection release with redirection to GERAN	277

6.3.2.3	RRC connection release with redirection to UTRAN TDD	278

6.3.2.4	RRC connection release with redirection to NR	278

6.3.2.5	RRC connection release with redirection to NR carrier subject to CCA	279

6.3.2.6	RRC connection release with redirection to NR Redcap	280

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	281

6.4.1	Introduction	281

6.4.2	Requirements	281

6.5	RRC Re-establishment for NB-IoT UEs	282

6.5.1	Introduction	282

6.5.2	Requirements	282

6.5.2.1	UE Re-establishment delay requirement in normal coverage	282

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	283

6.5A	RRC Re-establishment for NB-IoT UEs for Satellite Access	283

6.5A.1	Introduction	283

6.5A.2	Requirements	283

6.5A.2.1	UE Re-establishment delay requirement in normal coverage	284

6.5A.2.2	UE Re-establishment delay requirement in enhanced coverage	284

6.6	Random Access for UE category NB1	285

6.6.1	Introduction	285

6.6.2	Requirements	285

6.6.2.1	Correct behaviour when receiving Random Access Response reception	285

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	285

6.6.2.3	Correct behaviour when receiving a NACK on msg3	285

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	285

6.6.2.5	Correct behaviour when contention Resolution timer expires	285

6.6.2.6	MSG3-based channel quality report for UE Category NB1	286

6.6.3	Requirements for NPRACH configuration	286

6.6A	Random Access for UE category NB-IoT for Satellite Access	286

6.6A.1	Introduction	286

6.6A.2	Requirements	287

6.6A.2.1	Correct behaviour when receiving Random Access Response reception	287

6.6A.2.2	Correct behaviour when not receiving Random Access Response reception	287

6.6A.2.3	Correct behaviour when receiving a NACK on msg3	287

6.6A.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	287

6.6A.2.5	Correct behaviour when contention Resolution timer expires	287

6.6A.2.6	MSG3-based channel quality report for UE Category NB1	287

6.6A.3	Requirements for NPRACH configuration	288

6.7	RRC Re-establishment for Cat-M1 UEs	288

6.7.1	Introduction	288

6.7.2	Requirements	288

6.7.2.1	UE Re-establishment delay requirement for CEModeA	289

6.7.2.2	UE Re-establishment delay requirement for CEModeB	289

6.7A	RRC Re-establishment for Cat-M1 UEs for Satellite Access	290

6.7A.1	Introduction	290

6.7A.2	Requirements	290

6.7A.2.1	UE Re-establishment delay requirement for CEModeA	290

6.7A.2.2	UE Re-establishment delay requirement for CEModeB	291

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	291

6.8.1	Introduction	291

6.8.2	Requirements	292

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	292

6.8A	RRC Connection Release with Redirection for UE Category M1 for Satellite Access	292

6.8A.1	Introduction	292

6.8A.2	Requirements	293

6.8A.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	293

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	293

6.9.1	Introduction	293

6.9.2	Requirements	294

6.9A	RRC Connection Redirection to Non-anchor Carrier in NB-IoT for Satellite Access	294

6.9A.1	Introduction	294

7	Timing and signalling characteristics	295

7.1	UE transmit timing	295

7.1.1	Introduction	295

7.1.2	Requirements	296

7.2	UE timer accuracy	297

7.2.1	Introduction	297

7.2.2	Requirements	297

7.3	Timing Advance	297

7.3.1	Introduction	297

7.3.2	Requirements	297

7.3.2.1	Timing Advance adjustment delay	297

7.3.2.2	Timing Advance adjustment accuracy	298

7.4	Cell phase synchronization accuracy (TDD)	298

7.4.1	Definition	298

7.4.2	Minimum requirements	298

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	299

7.5.1	Introduction	299

7.5.2	eNodeB Synchronization Requirements	299

7.5.2.1	Synchronized E-UTRAN	299

7.5.2.2	Non-Synchronized E-UTRAN	299

7.6	Radio Link Monitoring	299

7.6.1	Introduction	299

7.6.2	Requirements	301

7.6.2.1	Minimum requirement when no DRX is used	301

7.6.2.2	Minimum requirement when DRX is used	301

7.6.2.3	Minimum requirement at transitions	303

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	303

7.6.2.5	Minimum requirement under IDC Interference	303

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	303

7.7.1	Introduction	303

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	304

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	305

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	306

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	307

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	308

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	309

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	310

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	310

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	310

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	311

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	312

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	313

7.7.14	SCell Activation Delay Requirement for Dormant SCell	313

7.7.15	SCell Hibernation Delay Requirement for Activated SCell	315

7.7.16	SCell Hibernation Delay Requirement for Deactivated SCell	315

7.7.17	SCell Deactivation Delay Requirement for Dormant SCell	317

7.7.18	Direct SCell Activation and Hibernation Delay Requirement	317

7.7.19	Direct SCell Activation and Hibernation Delay Requirement at RRC Reconfiguration during Handover	319

7.8	Interruptions with Carrier Aggregation	321

7.8.1	Introduction	321

7.8.2	Requirements	321

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	321

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	321

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	321

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	322

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	322

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	322

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	322

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	322

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	323

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	324

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	324

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	324

7.8.2.13	Interruptions at SRS carrier based switching	325

7.8.2.14	Interruptions at SCell activation and deactivation of dormant SCell for intra-band CA	326

7.8.2.15	Interruptions at SCell activation and deactivation of dormant SCell for inter-band CA	326

7.8.2.16	Interruptions at SCell activation and deactivation of multiple dormant SCells	326

7.8.2.17	Interruptions during CQI measurement on dormant SCell	326

7.8.2.18	Interruptions during RRM measurement on dormant SCell for intra-band CA	327

7.8.2.19	Interruptions during RRM measurement on dormant SCell for inter-band CA	327

7.8.2.20	Interruptions at SCell hibernation	327

7.8.2.21	Interruptions at direct SCell activation and hibernation	327

7.8.2.22	Interruptions during inter-RAT NR measurements without measurement gap	328

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	329

7.9.1	Introduction	329

7.9.2	Minimum Requirements for Interband Carrier Aggregation	329

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	329

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	329

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	330

7.10.1	Introduction	330

7.10.2	Requirements	330

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	330

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	330

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	330

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	331

7.11	Radio Link Monitoring for UE Category 0	331

7.11.1	Introduction	331

7.11.2	Requirements for FD-FDD and TDD	332

7.11.2.1	Minimum requirement when no DRX is used	332

7.11.2.2	Minimum requirement when DRX is used	333

7.11.2.3	Minimum requirement at transitions	333

7.11.3	Requirements for HD-FDD	334

7.11.3.1	Minimum requirement when no DRX is used	334

7.11.3.2	Minimum requirement when DRX is used	334

7.11.3.3	Minimum requirement at transitions	335

7.12	Interruptions with Dual Connectivity	335

7.12.1	Introduction	335

7.12.2	Requirements	335

7.12.2.1	Interruptions at PSCell addition/release	335

7.12.2.2	Interruptions at transitions between active and non-active during DRX	335

7.12.2.3	Interruptions at transitions from non-DRX to DRX	336

7.12.2.4	Interruptions at SCell addition/release	336

7.12.2.5	Interruptions at SCell activation/deactivation	336

7.12.2.6	Interruptions during measurements on SCC	337

7.12.2.7	Interruptions at SRS carrier based switching	337

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	338

7.13.1	Definition	338

7.13.2	Minimum requirements	338

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	338

7.14.1	Introduction	338

7.14.2	PSCell Addition Delay Requirement	338

7.14.3	PSCell Release Delay Requirement	339

7.15	Maximum Receive Timing Difference in Dual Connectivity	339

7.15.1	Introduction	339

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	339

7.16	Proximity-based Services	339

7.16.1	Introduction	339

7.16.2	Requirements	339

7.16.2.1	ProSe UE transmission timing	339

7.16.2.1.1	Serving cell or PCell as timing reference	340

7.16.2.1.2	SCell or non-serving cell as timing reference	340

7.16.3	Interruptions with ProSe	340

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	340

7.16.3.2	Interruptions at ProSe Direct Communication configuration	341

7.16.3.3	Interruptions during ProSe Direct Discovery	341

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	341

7.16.3.5	Interruptions during ProSe Direct Communication	342

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	342

7.16.4.1	Measurement and evaluation of selected cell	342

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	342

7.16.5	Selection / Reselection of ProSe relay UE	343

7.16.6	ProSe operation under deactivated SCell	343

7.17	Maximum Transmission Timing Difference in Dual Connectivity	343

7.17.1	Introduction	343

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	344

7.18.1	Introduction	344

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	344

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	344

7.19	Radio Link Monitoring for UE Category M1	344

7.19.1	Introduction	344

7.19.2	Requirements for FD-FDD and TDD CE mode A	345

7.19.2.1	Minimum requirement when no DRX is used	346

7.19.2.2	Minimum requirement when DRX is used	347

7.19.2.3	Minimum requirement at transitions	348

7.19.3	Requirements for HD-FDD with CE mode A	348

7.19.3.1	Minimum requirement when no DRX is used	349

7.19.3.2	Minimum requirement when DRX is used	349

7.19.3.3	Minimum requirement at transitions	350

7.19.4	Requirements for FD-FDD and TDD with CE mode B	350

7.19.4.1	Minimum requirement when no DRX is used	352

7.19.4.2	Minimum requirement when DRX is used	353

7.19.4.3	Minimum requirement at transitions	354

7.19.5	Requirements for HD-FDD with CE mode B	354

7.19.5.1	Minimum requirement when no DRX is used	354

7.19.5.2	Minimum requirement when DRX is used	355

7.19.5.3	Minimum requirement at transitions	356

7.19A	Radio Link Monitoring for UE Category M1 for Satellite Access	356

7.19A.1	Introduction	356

7.19A.2	Requirements for FD-FDD and CE mode A	356

7.19A.2.1	Minimum requirement when no DRX is used	358

7.19A.2.2	Minimum requirement when DRX is used	359

7.19A.2.3	Minimum requirement at transitions	360

7.19A.3	Requirements for HD-FDD with CE mode A	360

7.19A.3.1	Minimum requirement when no DRX is used	360

7.19A.3.2	Minimum requirement when DRX is used	361

7.19A.3.3	Minimum requirement at transitions	362

7.19A.4	Requirements for HD-FDD with CE mode B	362

7.19A.4.1	Minimum requirement when no DRX is used	362

7.19A.4.2	Minimum requirement when DRX is used	362

7.19A.4.3	Minimum requirement at transitions	363

7.20	UE transmit timing for NB-IoT	364

7.20.1	Introduction	364

7.20.2	Requirements	364

7.20A	UE transmit timing for NB-IoT for Satellite Access	364

7.20A.1	Introduction	364

7.20A.2	Requirements	365

7.21	 UE timer accuracy for NB-IoT	365

7.21.1	Introduction	365

7.21.2	Requirements	365

7.21A	UE timer accuracy for NB-IoT for Satellite Access	366

7.21A.1	Introduction	366

7.21A.2	Requirements	366

7.22	Timing Advance for NB-IoT	366

7.22.1	Introduction	366

7.22.2	Requirements	366

7.22.2.1	Timing Advance adjustment delay	366

7.22.2.2	Timing Advance adjustment accuracy	366

7.22A	Timing Advance for NB-IoT for Satellite Access	367

7.22A.1	Introduction	367

7.22A.2	Requirements	367

7.22A.2.1	Timing Advance adjustment delay	367

7.22A.2.2	Timing Advance adjustment accuracy	367

7.23	Radio Link Monitoring for Category NB1 UE	367

7.23.1	Introduction	367

7.23.2	Requirements for Category NB1 UE	367

7.23.2.1	Minimum requirement when no DRX is used	368

7.23.2.2	Minimum requirement when DRX is used	368

7.23.2.3	Minimum requirement at transitions	369

7.23A	Radio Link Monitoring for Category NB-IoT UE for Satellite Access	369

7.23A.1	Introduction	369

7.23A.2	Requirements for Category NB1 UE	369

7.23A.2.1	Minimum requirement when no DRX is used	370

7.23A.2.2	Minimum requirement when DRX is used	370

7.23A.2.3	Minimum requirement at transitions	371

7.24	UE transmit timing for Category M1	371

7.24.1	Introduction	371

7.24.2	Requirements	371

7.24A	UE transmit timing for Category M1 for Satellite Access	372

7.24A.1	Introduction	372

7.24A.2	Requirements	372

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	373

7.25.1	Definition	373

7.25.2	Minimum requirements	373

7.26	UE transmit timing for Category M2	374

7.26.1	Introduction	374

7.26.2	Requirements	374

7.27	UE timer accuracy for category M1	374

7.27.1	Introduction	374

7.27.2	Requirements	374

7.27A	UE timer accuracy for category M1 for Satellite Access	374

7.27A.1	Introduction	374

7.27A.2	Requirements	374

7.28	Timing Advance for Category M1	375

7.28.1	Introduction	375

7.28.2	Requirements	375

7.28A	Timing Advance for Category M1 for Satellite Access	375

7.28A.1	Introduction	375

7.28A.2	Requirements	375

7.28A.2.1	Timing Advance adjustment delay	375

7.28A.2.2	Timing Advance adjustment accuracy	375

7.29	Interruptions requirements with FeMBMS	375

7.29.1	Introduction	375

7.29.2	Requirements	375

7.30	Numerology switching delay requirements with FeMBMS	376

7.30.1	Introduction	376

7.30.2	Requirements	376

7.31	NR PSCell Addition and Release Delay for E-UTRA - NR Dual Connectivity	376

7.31.1	Introduction	376

7.31.2	NR PSCell Addition Delay Requirement	376

7.31.3	NR PSCell Release Delay Requirement	377

7.31A	Addition and Release Delay of NR PSCell Operating with CCA for E-UTRA - NR Dual Connectivity	377

7.31A.1	Introduction	377

7.31A.2	NR PSCell Addition Delay Requirement	377

7.31A.3	NR PSCell Release Delay Requirement	378

7.32	Interruptions with EN-DC	379

7.32.1	Introduction	379

7.32.2	Requirements	379

7.32.2.1	Interruptions at PSCell addition/release	379

7.32.2.2	Interruptions at transitions between active and non-active during DRX	379

7.32.2.3	Interruptions at transitions from non-DRX to DRX	380

7.32.2.4	Interruptions at SCell addition/release	380

7.32.2.5	Interruptions at SCell activation/deactivation	380

7.32.2.6	Interruptions during measurements on SCC	381

7.32.2.6.1	Interruptions during measurements on deactivated NR SCC	381

7.32.2.6.2	Interruptions during measurements on deactivated E-UTRA SCC	381

7.32.2.6.3	Interruptions during CQI measurements on dormant E-UTRA SCell	381

7.32.2.6.4	Interruptions during RRM measurements on dormant E-UTRA SCC	381

7.32.2.7	Interruptions at active BWP switching	381

7.32.2.8	Interruptions at SCell activation and deactivation of dormant SCell	382

7.32.2.9	Interruptions at SCell activation and deactivation of multiple dormant SCell	382

7.32.2.10	Interruptions at SCell hibernation	382

7.32.2.11	Interruptions at direct SCell activation and hibernation	382

7.32.2.12	DL Interruptions at UE switching between two uplink carriers	383

7.32.2.13	Interruptions at NR SRS carrier based switching	383

7.32.2.14	Interruptions at NR SCell dormancy	384

7.32.2.14.1	Interruptions due to NR SCell dormancy switch	384

7.32.2.14.2	Interruptions due to CSI and RRM measurements during SCell dormancy	384

7.32.2.15	Interruption during NR measurement with autonomous gaps	384

7.32.2.16	Interruptions at SRS carrier based switching	385

7.32.2.17	Interruptions at SCG activation/deactivation	385

7.32.2.18	Interruptions due to NR SRS antenna port switching	385

7.32.2.19	Interruptions at fast SCell activation/deactivation	386

7.32.2.20	Interruptions due to RRM measurements on deactivated NR SCG	386

7.32.2.21	Interruptions during RLM/BFD measurements on deactivated PScell	387

7.33	Maximum Transmit/Receive Timing Difference in Carrier Aggregation for sTTI and 1ms-TTI with 3 subframe HARQ processing	387

7.33.1	Introduction	387

7.33.2	Requirements	387

7.34	Void	387

7.35	Interruptions with SFTD measurements	387

7.35.1	Introduction	387

7.35.2	Requirements	387

7.36	Interruptions with NE-DC	388

7.36.1	Introduction	388

7.36.2	Requirements	388

7.36.2.1	Interruptions at transitions between active and non-active during DRX	388

7.36.2.2	Interruptions at transitions from non-DRX to DRX	388

7.36.2.3	Interruptions at SCell addition/release	388

7.36.2.4	Interruptions at SCell activation/deactivation	389

7.36.2.5	Interruptions during measurements on SCC	389

7.36.2.5.1	Interruptions during measurements on deactivated NR SCC	389

7.36.2.5.2	Interruptions during measurements on deactivated E-UTRA SCC	389

7.36.2.5.3	Interruptions during CQI measurements on dormant E-UTRA SCell	390

7.36.2.5.4	Interruptions during RRM measurements on dormant E-UTRA SCC	390

7.36.2.6	Interruptions at active BWP switching	390

7.36.2.7	Interruptions at SCell activation and deactivation of dormant SCell	391

7.36.2.8	Interruptions at SCell activation and deactivation of multiple dormant SCell	391

7.36.2.9	Interruptions at SCell hibernation	391

7.36.2.10	Interruptions at direct SCell activation and hibernation	391

7.36.2.11	Interruptions at NR SRS carrier based switching	391

7.36.2.12	Interruptions at NR SCell dormancy	392

7.36.2.12.1	Interruptions due to NR SCell dormancy switch	392

7.36.2.12.2	Interruptions due to CSI and RRM measurements during SCell dormancy	392

7.36.2.13	Interruption during E-UTRA measurement with autonomous gaps	393

7.36.2.14	Interruption during NR measurement with autonomous gaps	393

7.36.2.15	Interruptions at SRS carrier based switching	393

7.36.2.16	Interruptions due to NR SRS antenna port switching	394

7.37	Interruptions during NR measurement with autonomous gaps	395

7.37.1	Introduction	395

7.37.2	Requirements	395

7.38 SCG Activation and Deactivation Delay	395

7.38.1	Introduction	395

7.38.2	SCG Activation Delay Requirement	395

7.38.3	SCG Deactivation Delay Requirement	396

8	UE Measurements Procedures in RRC_CONNECTED State	396

8.1	General Measurement Requirements	396

8.1.1	Introduction	396

8.1.2	Requirements	397

8.1.2.1	UE measurement capability	397

8.1.2.1.1	Monitoring of multiple layers using gaps	405

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	406

8.1.2.1.1b	Monitoring of multiple layers using gaps (EN-DC)	407

8.1.2.1.1c	Monitoring of multiple layers using gaps (NE-DC)	409

8.1.2.1.1d	Monitoring of multiple layers using gaps (RedCap)	410

8.1.2.1.2	Network controlled small gap	410

8.1.2.2	E-UTRAN intra frequency measurements	412

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	412

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	417

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	421

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	422

8.1.2.2.5	E-UTRAN FDD intra-frequency measurements on carrier with FeMBMS/Unicast mixed cells	423

8.1.2.3	E-UTRAN inter frequency measurements	423

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	424

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	429

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	436

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	436

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	436

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	437

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	439

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	440

8.1.2.3.9	E-UTRAN FDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	441

8.1.2.3.10	E-UTRAN TDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	448

8.1.2.4	Inter RAT measurements	448

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	448

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	453

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	453

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	457

8.1.2.4.5	E-UTRAN FDD – GSM measurements	457

8.1.2.4.6	E-UTRAN TDD – GSM measurements	462

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	462

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	464

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	464

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	464

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	465

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	465

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	465

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	465

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	467

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	467

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	467

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	467

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	468

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	468

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	470

8.1.2.4.21	E-UTRAN FDD – NR measurements	470

8.1.2.4.21A	E-UTRAN FDD – NR measurements when CCA is used	474

8.1.2.4.22	E-UTRAN TDD – NR measurements	478

8.1.2.4.22A	E-UTRAN TDD – NR measurements when CCA is used	478

8.1.2.4.23	Void	478

8.1.2.4.24	Void	478

8.1.2.4.25	E-UTRAN FDD – NR SFTD Measurements	478

8.1.2.4.26	E-UTRAN TDD – NR SFTD Measurements	480

8.1.2.4.27 	E-UTRA FDD - NR measurements with autonomous gaps	480

8.1.2.4.28 	E-UTRA TDD - NR measurements with autonomous gaps	481

8.1.2.4.29	E-UTRAN FDD – NR measurements without measurement gap	481

8.1.2.4.30	E-UTRAN TDD – NR measurements without measurement gap	486

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	486

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	486

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	488

8.1.2.5.3	E-UTRAN FDD Intra-Frequency OTDOA Measurements for UE Category 1bis	489

8.1.2.5.4	E-UTRAN TDD Intra-Frequency OTDOA Measurements for UE Category 1bis	491

8.1.2.6.5	Void	493

8.1.2.6.6	Void	493

8.1.2.6.7	Void	493

8.1.2.6.8	Void	493

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	493

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	493

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	495

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	497

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	499

8.1.2.6.5	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	500

8.1.2.6.6	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	502

8.1.2.6.7	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	504

8.1.2.6.8	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	505

8.1.2.7	E-UTRAN E-CID Measurements	507

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	507

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	508

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	510

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	510

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	511

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	511

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	514

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	517

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	520

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	524

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	524

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	524

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	525

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	525

8.1.2.10	Void	526

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	526

8.2.1	Introduction	526

8.2.2	Requirements	526

8.3	Measurements for E-UTRA carrier aggregation	530

8.3.1	Introduction	530

8.3.2	Measurements of the primary component carrier	530

8.3.3	Measurements of a secondary component carrier	530

8.3.3.1	Measurements of a secondary component carrier with active SCell	530

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	531

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	531

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	532

8.3.3.3	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and activated SCell	534

8.3.3.4	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and deactivated SCell	534

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	534

8.4.1	Introduction	534

8.4.2	Measurements on the primary component carrier	535

8.4.3	Measurements on a secondary component carrier	536

8.4.4	Measurements on both primary component carrier and a secondary component carrier	536

8.4.5	Measurements on different secondary component carriers	537

8.5	Measurements for UE category 0	538

8.5.1	Introduction	538

8.5.2 	Requirements	539

8.5.2.1 	E-UTRAN intra frequency measurements	539

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	539

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	542

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	544

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	548

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	548

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	549

8.6	Discovery signal measurements	550

8.6.1	Introduction	550

8.6.2	Requirements for CRS based discovery signal measurements	550

8.6.2.1	E-UTRAN intra frequency measurements	550

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	550

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	553

8.6.2.2	E-UTRAN inter frequency measurements	555

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	556

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	558

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	561

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	561

8.6.3	Requirements for CSI-RS based discovery signal measurements	561

8.6.3.1	E-UTRAN intra frequency measurements	561

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	562

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	564

8.6.3.2	E-UTRAN inter frequency measurements	566

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	567

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	569

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	572

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	572

8.7	Discovery signal measurements for E-UTRA carrier aggregation	572

8.7.1	Introduction	572

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	573

8.7.2.1	Measurements of the primary component carrier	573

8.7.2.2	Measurements of a secondary component carrier	573

8.7.2.3	Measurements of a secondary component carrier with active SCell	573

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	573

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	573

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	574

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	576

8.7.3.1	Measurements of the primary component carrier	576

8.7.3.2	Measurements of a secondary component carrier	576

8.7.3.3	Measurements of a secondary component carrier with active SCell	576

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	576

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	576

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	578

8.8	Measurements for E-UTRA dual connectivity	579

8.8.1	Introduction	579

8.8.2	Intra-frequency measurements requirements on PCell	579

8.8.3	Intra-frequency measurements requirements on PSCell	580

8.8.4	Inter-frequency and inter-RAT measurement requirements	580

8.8.5	Intra-frequency measurements with autonomous gaps	580

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	580

8.8.5.2	ECGI reporting delay	581

8.8.6	Inter-frequency measurements with autonomous gaps	581

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	581

8.8.6.2	ECGI reporting delay	582

8.8.7	SSTD Measurements	582

8.8.7.1	Introduction	582

8.8.7.2	SSTD Measurement requirements	582

8.8.7.3	SSTD Measurement Reporting Delay	583

8.8.8	Intra-frequency measurements requirements on SCell	583

8.9	MBSFN Measurements	583

8.9.1	Introduction	583

8.9.2	MBSFN RSRP Measurements	583

8.9.3	MBSFN RSRQ Measurements	584

8.9.4	MCH BLER Measurements	584

8.10	Proximity-based Services	584

8.10.1	Introduction	584

8.10.2	Requirements	584

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	584

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	585

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	586

8.11.1	Introduction	586

8.11.2	CRS based discovery signal measurements	586

8.11.2.1	E-UTRAN intra-frequency measurements	586

8.11.2.1.1	Requirements	586

8.11.2.1.1.1	Requirements when no DRX is used	586

8.11.2.1.1.1.1	Measurement Reporting Requirements	588

8.11.2.1.1.2	Requirements when DRX is used	589

8.11.2.1.1.2.1	Measurement Reporting Requirements	591

8.11.2.2	E-UTRAN inter-frequency measurements	591

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	591

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	595

8.11.3	CSI-RS based discovery signal measurements	595

8.11.3.1	E-UTRAN intra-frequency measurements	595

8.11.3.1.1	Requirements	596

8.11.3.1.1.1	Requirements when no DRX is used	596

8.11.3.1.1.1.1	Measurement Reporting Requirements	597

8.11.3.1.1.2	Requirements when DRX is used	598

8.11.3.1.1.2.1	Measurement Reporting Requirements	599

8.11.3.2	E-UTRAN inter-frequency measurements	600

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	600

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	604

8.11.4	RSSI measurements	604

8.11.4.1	E-UTRAN intra-frequency measurements	604

8.11.4.2	E-UTRAN inter-frequency measurements	604

8.11.5	Channel occupancy measurements	604

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	604

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	605

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	605

8.12.1	Introduction	605

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	605

8.12.2.1	Introduction	605

8.12.2.2	Measurements of a secondary component carrier	605

8.12.2.3	Measurements of a secondary component carrier with active SCell	605

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	605

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	606

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	608

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	611

8.12.3.1	Introduction	611

8.12.3.2	Measurements of a secondary component carrier	611

8.12.3.3	Measurements of a secondary component carrier with active SCell	611

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	611

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	611

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	613

8.13	Measurements for UE Category M1	615

8.13.1	Introduction	615

8.13.2	Requirements for UE category M1 with CE mode A	615

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	616

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	616

8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	620

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	623

8.13.2.2	Void	627

8.13.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	627

8.13.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	627

8.13.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	630

8.13.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	632

8.13.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	633

8.13.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	633

8.13.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	635

8.13.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	637

8.13.2.5	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	638

8.13.2.5.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	638

8.13.2.5.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	638

8.13.2.5.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	639

8.13.2.5.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	639

8.13.2.5.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	639

8.13.2.5.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	640

8.13.2.5.7	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	640

8.13.2.5.8	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	641

8.13.2.5.9	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	642

8.13.2.6	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	642

8.13.2.6.1	E-UTRAN FDD - FDD inter frequency measurements	643

8.13.2.6.2	E-UTRAN inter-frequency measurements for HD-FDD	647

8.13.2.6.3	E-UTRAN TDD inter frequency measurements	649

8.13.2.7	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	653

8.13.2.8	Channel quality report for UE Category M1 in connected mode with CE mode A	653

8.13.3	Requirements for UE category M1 with CE mode B	654

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	654

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	655

8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	659

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	662

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	667

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	668

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	668

8.13.3.2	Void	669

8.13.3.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	669

8.13.3.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	669

8.13.3.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	672

8.13.3.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	674

8.13.3.4	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	675

8.13.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	675

8.13.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	675

8.13.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	675

8.13.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	676

8.13.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	676

8.13.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	677

8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	677

8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements	677

8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD	682

8.13.3.5.3	E-UTRAN TDD inter frequency measurements	684

8.13.3.6	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	689

8.13.3.7	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	689

8.13.3.7.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	689

8.13.3.7.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	691

8.13.3.7.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	694

8.13.3.8	Channel quality report for UE Category M1 in connected mode with CE mode B	694

8.13A	Measurements for UE Category M1 for Satellite Access	695

8.13A.1	Introduction	695

8.13A.2	Requirements for UE category M1 with CE mode A	695

8.13A.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	696

8.13A.2.1.1	E-UTRAN FDD intra frequency measurements	696

8.13A.2.1.1.1	E-UTRAN intra frequency measurements when no DRX is used	696

8.13A.2.1.1.2	E-UTRAN intra frequency measurements when DRX is used	698

8.13A.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	700

8.13A.2.1.2.1	E-UTRAN intra frequency measurements when no DRX is used	700

8.13A.2.1.2.2	E-UTRAN intra frequency measurements when DRX is used	700

8.13A.2.2	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	702

8.13A.2.2.1	E-UTRAN FDD - FDD inter frequency measurements	702

8.13A.2.2.1.1	E-UTRAN FDD - FDD inter frequency measurements when no DRX is used	702

8.13A.2.2.1.2	E-UTRAN inter frequency measurements when DRX is used	704

8.13A.2.2.2	E-UTRAN inter-frequency measurements for HD-FDD	706

8.13A.2.2.2.1	E-UTRAN inter-frequency measurements when no DRX is used	706

8.13A.2.2.2.2	E-UTRAN inter frequency measurements when DRX is used	706

8.13A.2.3	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	708

8.13A.2.4	Channel quality report for UE Category M1 in connected mode with CE mode A	709

8.13A.3	Requirements for UE category M1 with CE mode B	709

8.13A.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	710

8.13A.3.1.1	E-UTRAN FDD intra frequency measurements	710

8.13A.3.1.1.1	E-UTRAN intra frequency measurements when no DRX is used	710

8.13A.3.1.1.2	E-UTRAN intra frequency measurements when DRX is used	712

8.13A.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	714

8.13A.3.1.2.1	E-UTRAN intra frequency measurements when no DRX is used	714

8.13A.3.1.2.2	E-UTRAN intra frequency measurements when DRX is used	715

8.13A.3.2	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	717

8.13A.3.2.1	E-UTRAN FDD - FDD inter frequency measurements	717

8.13A.3.2.1.1	E-UTRAN FDD - FDD inter frequency measurements when no DRX is used	717

8.13A.3.2.1.2	E-UTRAN inter frequency measurements when DRX is used	719

8.13A.3.2.2	E-UTRAN inter-frequency measurements for HD-FDD	721

8.13A.3.2.2.1	E-UTRAN inter-frequency measurements when no DRX is used	721

8.13A.3.2.2.2	E-UTRAN inter frequency measurements when DRX is used	721

8.13A.3.3	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	723

8.13A.3.4	Channel quality report for UE Category M1 in connected mode with CE mode B	724

8.14	Measurements for UE category NB1	724

8.14.1	Introduction	724

8.14.2	NB-IoT intra frequency measurements under normal coverage	724

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	724

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	724

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	725

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	725

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	725

8.14.4	Connected mode channel quality report for UE Category NB1	725

8.14.5	Connected mode channel quality report for UE Category NB2 supporting 16QAM	725

8.14.6	NB-IoT neighbour cell measurements	726

8.14.6.1	Introduction	726

8.14.6.2	Requirements	726

8.14.6.3	Intra-frequency neighbour cell measurements	726

8.14.6.4	Inter-frequency neighbour cell measurements	727

8.14.6.5	 Requirements for monitoring multiple carriers	728

8.14A	Measurements for UE category NB-IoT for Satellite Access	728

8.14A.1	Introduction	728

8.14A.2	NB-IoT intra frequency measurements under normal coverage	729

8.14A.2.1	NB-IoT intra frequency measurements when no DRX is used	729

8.14A.2.2	NB-IoT intra frequency measurements when DRX is used	729

8.14A.3	NB-IoT intra frequency measurements under enhanced coverage	729

8.14A.3.1	NB-IoT intra frequency measurements when no DRX is used	729

8.14A.3.2	NB-IoT intra frequency measurements when DRX is used	729

8.14A.4	Connected mode channel quality report for UE Category NB1	729

8.15	Void	732

8.16	Measurements for UE Category M2	732

8.16.1	Introduction	732

8.16.2	Requirements for UE category M2 with CE mode A	733

8.16.2.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	733

8.16.2.1.1	UE Rx-Tx Measurement Reporting Delay	733

8.16.2.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	734

8.16.2.2.1	UE Rx-Tx Measurement Reporting Delay	734

8.16.2.2a	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	735

8.16.2.2a.1	UE Rx-Tx Measurement Reporting Delay	735

8.16.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	735

8.16.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	736

8.16.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	738

8.16.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	741

8.16.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	741

8.16.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	741

8.16.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	743

8.16.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	746

8.16.3	Requirements for UE category M2 with CE mode B	746

8.16.3.1	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	746

8.16.3.1.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	746

8.16.3.1.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	749

8.16.3.1.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	752

8.16.3.2	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	752

8.16.3.2.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	752

8.16.3.2.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	754

8.16.3.2.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	757

8.17	Measurements for E-UTRA – NR Dual Connectivity	757

8.17.1	Introduction	757

8.17.1.1	Measurement Gap Sharing	757

8.17.1A	Intrafrequency Measurements	758

8.17.2	SFTD Measurements	758

8.17.2.1	Introduction	758

8.17.2.2	SFTD Measurement requirements	758

8.17.2.2.a	SFTD Measurement requirements with CCA on target frequency	759

8.17.2.3	SFTD Measurement Reporting Delay	760

8.17.3	E-UTRA Inter-frequency Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	760

8.17.3.1	Introduction	760

8.17.3.2	E-UTRAN FDD inter frequency measurements	761

8.17.3.2.1	E-UTRAN FDD inter frequency measurements when no DRX is used	761

8.17.3.2.2	E-UTRAN FDD inter frequency measurements when DRX is used	762

8.17.3.3	E-UTRAN TDD inter frequency measurements	764

8.17.3.3.1	E-UTRAN TDD inter frequency measurements when no DRX is used	764

8.17.3.3.2	E-UTRAN TDD inter frequency measurements when DRX is used	766

8.17.4	E-UTRA Inter-RAT NR Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	768

8.17.4.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	768

8.17.4.1.1	NR Inter-RAT cell identification	768

8.17.4.1.2	NR Inter-RAT measurement	770

8.17.4.1.3	NR Inter-RAT measurement reporting	770

8.17.4.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	771

8.17.4A	E-UTRA Inter-RAT NR Measurements when CCA is used when Configured with E-UTRA-NR Dual Connectivity Operation	771

8.17.4A.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	771

8.17.4A.1.1	NR Inter-RAT cell identification	772

8.17.4A.1.2	NR Inter-RAT measurement	773

8.17.4A.1.3	NR Inter-RAT measurement reporting	774

8.17.4A.1.4	NR inter-RAT RSSI measurements	775

8.17.4A.1.5	NR inter-RAT channel occupancy measurements	775

8.17.4A.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	776

8.17.5	E-UTRAN FDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	776

8.17.5.1	Introduction	776

8.17.5.2	E-UTRAN FDD – UTRAN FDD measurements when no DRX is used	776

8.17.5.2.1	Identification of a new UTRA FDD cell	776

8.17.5.2.2	Enhanced UTRA FDD cell identification requirements	776

8.17.5.2.3	UE UTRA FDD CPICH measurement capability	777

8.17.5.2.4	Periodic Reporting	777

8.17.5.2.5	Event Triggered Reporting	777

8.17.5.2.6	Event-triggered Periodic Reporting	778

8.17.5.3	E-UTRAN FDD – UTRAN FDD measurements when DRX is used	778

8.17.5.3.1	Periodic Reporting	779

8.17.5.3.2	Event Triggered Reporting	779

8.17.5.3.3	Event-triggered Periodic Reporting	780

8.17.6	E-UTRAN TDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	780

8.17.7	E-UTRAN FDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	780

8.17.7.1	Introduction	780

8.17.7.2	Identification of a new UTRA FDD cell for SON	780

8.17.7.2.1	Requirements when no DRX is used	780

8.17.7.2.2	Requirements when DRX is used	781

8.17.7.2.3	Reporting Delay	781

8.17.8	E-UTRAN TDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	782

8.17.9	E-UTRAN TDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	782

8.17.9.1	Introduction	782

8.17.9.2	E-UTRAN FDD – UTRAN TDD measurements when no DRX is used	782

8.17.9.2.1	Identification of a new UTRA FDD cell	782

8.17.9.2.2	Enhanced UTRA TDD cell identification requirements	782

8.17.9.2.3	UE UTRA TDD P-CCPCH RSCP measurement capability	783

8.17.9.2.4	Periodic Reporting	783

8.17.9.2.5	Event Triggered Reporting	783

8.17.9.2.6	Event-triggered Periodic Reporting	784

8.17.9.3	E-UTRAN TDD – UTRAN TDD measurements when DRX is used	784

8.17.9.3.1	Periodic Reporting	785

8.17.9.3.2	Event Triggered Reporting	785

8.17.9.3.3	Event-triggered Periodic Reporting	785

8.17.10	E-UTRAN FDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	786

8.17.11	E-UTRAN TDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	786

8.17.11.1	Introduction	786

8.17.11.2	Identification of a new UTRA TDD cell for SON	786

8.17.11.2.1	Requirements when no DRX is used	786

8.17.11.2.2	Requirements when DRX is used	786

8.17.11.2.3	Reporting Delay	787

8.17.12	E-UTRAN FDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	787

8.17.13	E-UTRAN FDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	788

8.17.13.1	Introduction	788

8.17.13.2	E-UTRAN FDD – GSM measurements when no DRX is used	788

8.17.13.2.1	GSM carrier RSSI	788

8.17.13.2.2	BSIC verification	788

8.17.13.2.3	Enhanced BSIC verification	790

8.17.13.2.4	Periodic Reporting	790

8.17.13.2.5	Event Triggered Reporting	790

8.17.13.2.6	Event-triggered Periodic Reporting	791

8.17.13.3	E-UTRAN FDD – GSM measurements when DRX is used	791

8.17.13.3.1	GSM carrier RSSI	791

8.17.13.3.2	BSIC verification	791

8.17.13.3.3	Periodic Reporting	793

8.17.13.3.4	Event Triggered Reporting	793

8.17.13.3.5	Event-triggered Periodic Reporting	793

8.17.14	E-UTRAN TDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	793

8.17.15	E-UTRAN Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	793

8.17.15.1	E-UTRAN FDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	793

8.17.15.1.1	RSTD Measurement Reporting Delay	794

8.17.15.2	E-UTRAN TDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	794

8.17.15.2.1	RSTD Measurement Reporting Delay	794

8.17.15.3	E-UTRAN TDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	794

8.17.15.3.1	RSTD Measurement Reporting Delay	795

8.17.15.4	E-UTRAN FDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	795

8.17.15.4.1	RSTD Measurement Reporting Delay	795

8.17.16	E-UTRAN intra-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	796

8.17.16.1	Introduction	796

8.17.16.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	796

8.17.16.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	796

8.17.17	E-UTRAN inter-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	796

8.17.17.1	Introduction	796

8.17.17.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	796

8.17.17.3	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	796

8.17.17.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	796

8.17.17.5	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	796

8.17.18	E-UTRA FDD - NR CGI measurements with autonomous gaps	796

8.17.19	E-UTRA TDD - NR CGI measurements with autonomous gaps	796

8.18	Measurements for non-BL/CE UE	796

8.18.1	Introduction	796

8.18.2	Requirements for non-BL/CE UE with CE Mode B	797

8.18.2.1	E-UTRAN intra frequency measurements	797

8.18.2.1.1	E-UTRAN FDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	797

8.18.2.1.2	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD non-BL/CE with CE Mode B	797

8.18.2.1.3	E-UTRAN TDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	798

8.19	Measurements for NR – E-UTRA Dual Connectivity	799

8.19.1	Introduction	799

8.19.2	Intra-frequency Measurements	799

8.19.3	Inter-frequency Measurements	799

8.19.4	Void	799

8.19.5	Intra-frequency E-CID Measurements	799

8.19.6	Intra-frequency measurements with autonomous gaps	800

8.19.6.1	Introduction	800

8.19.6.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	800

8.19.6.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	800

8.19.7	Inter-frequency measurements with autonomous gaps	800

8.19.7.1	Introduction	800

8.19.7.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	800

8.19.7.3	E-UTRAN TDD-FDD inter frequency measurements with autonomous gaps	800

8.19.7.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	800

8.19.7.5	E-UTRAN FDD-TDD inter frequency measurements with autonomous gaps	800

8.20	Inter-RAT NR Measurements for RedCap UE	800

8.20.1	Introduction	800

8.20.2	E-UTRAN FDD – NR measurements	800

8.20.2.1	Identification of a new NR cell	800

8.20.2.2	Periodic Reporting	804

8.20.2.3	Event Triggered Reporting	804

8.20.2.4	Event-triggered Periodic Reporting	804

8.20.3	E-UTRAN TDD – NR measurements	804

9	Measurements performance requirements for UE	805

9.1	E-UTRAN measurements	805

9.1.1	Introduction	805

9.1.2	Intra-frequency RSRP Accuracy Requirements	805

9.1.2.1	Absolute RSRP Accuracy	805

9.1.2.2	Relative Accuracy of RSRP	806

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	807

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	807

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	808

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	809

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	810

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	811

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	812

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	812

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	812

9.1.2B	Intra-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	813

9.1.2B.1	Introduction	813

9.1.2B.2	Intra-frequency Absolute RSRP Accuracy for CA Idle Mode Measurements	813

9.1.3	Inter-frequency RSRP Accuracy Requirements	814

9.1.3.1	Absolute RSRP Accuracy	814

9.1.3.2	Relative Accuracy of RSRP	815

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	816

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	816

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	817

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	817

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	818

9.1.3B	Inter-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	819

9.1.3B.1	Introduction	819

9.1.3B.2	Inter-frequency Absolute RSRP Accuracy for Overlapping Carrier	819

9.1.3B.3	Inter-frequency Absolute RSRP Accuracy for Overlapping and Non-overlapping Carrier	819

9.1.4	RSRP Measurement Report Mapping	820

9.1.5	Intra-frequency RSRQ Accuracy Requirements	820

9.1.5.1	Absolute RSRQ Accuracy	820

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	821

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	822

9.1.5.4	Absolute WB-RSRQ Accuracy	823

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	824

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	825

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	825

9.1.5B	Intra-frequency RSRQ Accuracy requirements for CA Idle Mode Measurements	825

9.1.5B.1	Introduction	825

9.1.5B.2	Intra-frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements	825

9.1.6	Inter-frequency RSRQ Accuracy Requirements	826

9.1.6.1	Absolute RSRQ Accuracy	826

9.1.6.2	Relative Accuracy of RSRQ	827

9.1.6.3	Absolute WB-RSRQ Accuracy	828

9.1.6.4	Relative WB-RSRQ Accuracy	828

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	829

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	830

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	830

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	830

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	831

9.1.6B	Inter-frequency absolute RSRQ Accuracy requirements for CA Idle Mode Measurements	832

9.1.6B.1	Introduction	832

9.1.6B.2	Inter-frequency Absolute RSRQ Accuracy for Overlapping Carrier	832

9.1.6B.3	Inter-frequency absolute RSRQ Accuracy for Overlapping and Non-overlapping Carrier	832

9.1.7	RSRQ Measurement Report Mapping	833

9.1.8	Power Headroom	833

9.1.8.1	Period	834

9.1.8.2	Reporting Delay	834

9.1.8.3	Void	834

9.1.8.4	Report Mapping	834

9.1.8A	Power Headroom for UE category M1 for satellite access	834

9.1.8A.1	Period	835

9.1.8A.2	Reporting Delay	835

9.1.8A.3	Void	835

9.1.8A.4	Report Mapping	835

9.1.9	UE Rx – Tx time difference	835

9.1.9.1	Measurement Requirement	835

9.1.9.2	Measurement Report mapping	836

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	837

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	838

9.1.10	Reference Signal Time Difference (RSTD)	839

9.1.10.1	Intra-Frequency Accuracy Requirement	839

9.1.10.2	Inter-Frequency Accuracy Requirement	840

9.1.10.3	RSTD Measurement Report Mapping	842

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	842

9.1.10.5	Intra-Frequency Accuracy Requirement for UE Category 1bis	843

9.1.10.6	Inter-Frequency Accuracy Requirement for UE Category 1bis	844

9.1.11	Carrier aggregation measurement accuracy	845

9.1.11.1	Primary component carrier accuracy requirement	846

9.1.11.2	Secondary component carrier accuracy requirement	846

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	846

9.1.11.4	Secondary component carrier relative accuracy requirement	846

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	846

9.1.13	Measurement accuracy for UE category 0	847

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	847

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	847

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	848

9.1.14	Accuracy requirements for Discovery Signal Measurements	849

9.1.14.1	Introduction	849

9.1.14.2	RSRP measurements in discovery signal occasions	849

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	849

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	849

9.1.14.3.1.1	Absolute CSI-RSRP measurement requirements	849

9.1.14.3.1.2	Relative CSI-RSRP measurement requirements	850

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	851

9.1.14.3.2.1	Absolute CSI-RSRP measurement requirements	851

9.1.14.3.2.2	Relative CSI-RSRP measurement requirements	851

9.1.14.3.3	CSI-RSRP measurement report mapping	852

9.1.14.4	RSRQ measurements in discovery signal occasions	852

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	852

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	853

9.1.15.1.1	Primary component carrier accuracy requirement	853

9.1.15.1.2	Secondary component carrier accuracy requirement	853

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	853

9.1.15.1.4	Secondary component carrier relative accuracy requirement	853

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	853

9.1.15.2.1	Primary component carrier accuracy requirement	853

9.1.15.2.2	Secondary component carrier accuracy requirement	853

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	853

9.1.15.2.4	Secondary component carrier relative accuracy requirement	853

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	854

9.1.17	RS-SINR Measurements	854

9.1.17.1	Measurement Report Mapping	854

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	855

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	855

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	855

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	855

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	856

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	857

9.1.18.1	Introduction	857

9.1.18.2	RSRP measurements	857

9.1.18.2.1	RSRP measurement report mapping	857

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	857

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	858

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	858

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	859

9.1.18.3	RSRQ measurements	859

9.1.18.3.1	RSRQ measurement report mapping	859

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	859

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	860

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	860

9.1.18.4	CSI-RSRP measurements	861

9.1.18.4.1	CSI-RSRP measurement report mapping	861

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	861

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	861

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	862

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	862

9.1.18.5	RSSI measurements	863

9.1.18.5.1	RSSI measurement report mapping	863

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	863

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	864

9.1.18.6	Channel occupancy measurements	864

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	864

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	864

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	865

9.1.19.1	Introduction	865

9.1.19.2	Accuracy requirements for measurements on SCC	865

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	865

9.1.19.4	Relative accuracy requirements for measurements on SCC and PCC	865

9.1.20	SFN and Subframe Time Difference (SSTD)	866

9.1.20.1	SSTD Accuracy Requirement	866

9.1.20.2	SSTD Measurement Report Mapping	866

9.1.21	Measurement accuracy for UE category M1	867

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	867

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	869

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	870

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	872

9.1.21.5	RSRP Measurement Report Mapping	873

9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	873

9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	874

9.1.21.8	RSRQ Measurement Report Mapping	875

9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	875

9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	876

9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	877

9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	878

9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	879

9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	880

9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	881

9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	882

9.1.21.17	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	883

9.1.21.18	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	884

9.1.21.19	UE RX-TX time difference Accuracy Requirement for Cat-M1	885

9.1.21.20	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	886

9.1.21.21	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	889

9.1.21.22	Downlink Channel Report Mapping for UE Category M1	892

9.1.21.23	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode A	892

9.1.21.24	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode B	895

9.1.21A	Measurement accuracy for UE category M1 for satellite access	896

9.1.21A.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	897

9.1.21A.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	898

9.1.21A.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	899

9.1.21A.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	901

9.1.21A.5	RSRP Measurement Report Mapping	902

9.1.21A.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	902

9.1.21A.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	903

9.1.21A.8	RSRQ Measurement Report Mapping	904

9.1.21A.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	904

9.1.21A.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	905

9.1.21A.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	906

9.1.21A.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	907

9.1.21A.13	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	908

9.1.21A.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	909

9.1.21A.15	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	910

9.1.21A.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	911

9.1.21A.17	Downlink Channel Report Mapping for UE Category M1	912

9.1.21A.18	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode A	912

9.1.21A.19	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode B	914

9.1.22	Measurement accuracy for UE Category NB1	915

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	915

9.1.22.2	Void	916

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1	916

9.1.22.4	Void	917

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	917

9.1.22.6	Void	918

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	918

9.1.22.8	Void	919

9.1.22.9	NRSRP Measurement Report Mapping	919

9.1.22.10	Intra-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	919

9.1.22.11	Inter-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	920

9.1.22.12	Intra-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	921

9.1.22.13	Inter-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	922

9.1.22.14	NRSRQ Measurement Report Mapping	923

9.1.22.15	MSG3-based Measurement Report Mapping for UE Category NB1	924

9.1.22.16	Downlink Channel Quality Measurement Accuracy for UE Category NB1	924

9.1.22.17	Channel quality reporting for UE Category NB2 with 16-QAM	925

9.1.22A	Measurement accuracy for UE Category NB1 for satellite access	925

9.1.22A.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	925

9.1.22A.2	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1	926

9.1.22A.3	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	927

9.1.22A.4	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	928

9.1.22A.5	NRSRP Measurement Report Mapping	929

9.1.22A.6	NRSRQ Measurement Report Mapping	929

9.1.22A.7	MSG3-based Measurement Report Mapping for UE Category NB1	929

9.1.22A.8	Downlink Channel Quality Measurement Accuracy for UE Category NB1	929

9.1.23	Power Headroom for UE Category NB1	930

9.1.23.1	Period	930

9.1.23.2	Reporting Delay	930

9.1.23.3	Report Mapping for UE Category NB1	930

9.1.23.3.1	Void	932

9.1.23.3.2	Void	932

9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6	932

9.1.23A	Power Headroom for UE Category NB1 for Satellite Access	933

9.1.23A.1	Period	933

9.1.23A.2	Reporting Delay	933

9.1.23A.3	Report Mapping for UE Category NB1 for Satellite Access	934

9.1.23A.3.1	Void	934

9.1.23A.3.2	Void	934

9.1.23A.4	Report Mapping for UE Category NB1 for UE Power Class 6 for Satellite Access	934

9.1.24	Void	934

9.1.25	Measurement accuracy for UE category M2	934

9.1.25.1	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	934

9.1.25.2	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	935

9.1.25.3	UE RX-TX time difference Accuracy Requirement for Cat-M2	936

9.1.25.4	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	937

9.1.25.5	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	939

9.1.26	Measurement Accuracy for non-BL CE UE	940

9.1.26.1	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	941

9.1.26.2	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	943

9.1.26.3	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	943

9.1.26.4	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	945

9.1.26.5	RSRP Measurement Report Mapping	946

9.1.26.6	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	946

9.1.26.7	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	946

9.1.26.8	RSRQ Measurement Report Mapping	947

9.1.26.9	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	947

9.1.26.10	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	948

9.1.26.11	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	949

9.1.26.12	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	950

9.1.26.13	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	951

9.1.26.14	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode A	951

9.1.26.15	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	951

9.1.26.16	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode B	952

9.1.27	SFN and frame Timing Difference (SFTD)	953

9.1.27.1	SFTD Accuracy Requirement	953

9.1.28	SFN and Frame Timing Difference (SFTD) under CCA	955

9.1.28.1	SFTD Accuracy Requirement under CCA	955

9.2	UTRAN FDD Measurements	956

9.2.1	UTRAN FDD CPICH RSCP	956

9.2.2	Void	957

9.2.3	UTRAN FDD CPICH Ec/No	957

9.3	UTRAN TDD Measurements	957

9.3.1	UTRAN TDD P-CCPCH RSCP	958

9.3.2	Void	958

9.3.3	Void	958

9.4	GSM Measurements	958

9.4.1	GSM carrier RSSI	958

9.5	CDMA2000 1x RTT Measurements	958

9.5.1	CDMA2000 1x RTT Pilot Strength	958

9.6	PCMAX,c	959

9.6.1	Report Mapping	959

9.6.2 	Estimation Period	959

9.6.3	Reporting Delay	959

9.7	IEEE802.11 Measurements	959

9.7.1	WLAN RSSI	959

9.7.2	WLAN RSSI Measurement Report Mapping	959

9.8	MBSFN Measurements	960

9.8.1	Introduction	960

9.8.2	MBSFN RSRP	960

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	960

9.8.2.2	MBSFN RSRP measurement report mapping	961

9.8.2.3	MBSFN RSRP measurement report mapping for 7.5 kHz subcarrier spacing	961

9.8.2.4	MBSFN RSRP measurement report mapping for 1.25 kHz subcarrier spacing	962

9.8.2.5	MBSFN RSRP measurement report mapping for 2.5 kHz subcarrier spacing	962

9.8.2.6	MBSFN RSRP measurement report mapping for 370.37Hz subcarrier spacing	962

9.8.3	MBSFN RSRQ	963

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	963

9.8.3.2	MBSFN RSRQ measurement report mapping	963

9.8.3.3	MBSFN RSRQ measurement report mapping for 7.5 kHz subcarrier spacing	964

9.8.3.4	MBSFN RSRQ measurement report mapping for 1.25 kHz subcarrier spacing	964

9.8.3.5	MBSFN RSRQ measurement report mapping for 2.5 kHz subcarrier spacing	964

9.8.3.6	MBSFN RSRQ measurement report mapping for 370.37 kHz subcarrier spacing	965

9.8.4	MCH BLER	965

9.8.4.1	Measurement report mapping for MCH BLER	965

9.8.4.2	Measurement report mapping for MCH Block Number	966

9.9	ProSe Measurements	967

9.9.1	Introduction	967

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	967

9.9.2.1	Absolute S-RSRP Accuracy	967

9.9.2.2	Relative Accuracy of S-RSRP	968

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	969

9.9.3.1	Absolute SD-RSRP Accuracy	969

9.9.3.2	Relative Accuracy of SD-RSRP	969

9.10	V2X Measurements	970

9.10.1	Introduction	970

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	970

9.10.2.1	Absolute S-RSRP Accuracy	970

9.10.2.2	Relative Accuracy of S-RSRP	971

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	971

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	971

9.10.4	S-RSSI Measurement Accuracy Requirements	972

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	972

9.10.4.2 Intra-frequency relative S-RSSI measurement accuracy requirements	972

9.11	NR Measurements	973

9.11.1	NR SS-RSRP Measurements	973

9.11.1A	NR SS-RSRP Measurements for DC Idle Mode Measurements	973

9.11.2	NR SS-RSRQ Measurements	974

9.11.2A	NR SS-RSRQ Measurements for DC Idle Mode Measurements	974

9.11.3	NR SS-SINR Measurements	974

9.11.4	NR SS-RSRP Measurements under CCA	975

9.11.5	NR SS-RSRQ Measurements under CCA	975

9.11.6	NR SS-SINR Measurements under CCA	975

9.11.7	NR RSSI Measurements under CCA	975

9.11.8	NR Channel Occupancy Measurements under CCA	976

10	Measurements Performance Requirements for E-UTRAN	976

10.1	Received Interference Power	976

10.1.1	Absolute accuracy requirement	976

10.1.2	Relative accuracy requirement	976

10.1.3	Received Interference Power measurement report mapping	977

10.2	Angle of Arrival (AOA)	977

10.2.1	Range/mapping	977

10.3	Timing Advance (TADV)	977

10.3.1	Report mapping	977

11	ProSe Requirements in Any Cell Selection state	978

11.1	Introduction	978

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	978

11.2.1	Introduction	978

11.2.2	ProSe UE transmission timing	978

11.3	Initiation/Cease of SLSS Transmissions	979

11.3.1	Introduction	979

11.3.2	Requirements	979

11.4	Measurements for ProSe in Any Cell Selection State	979

11.4.1	Introduction	979

11.4.2	Requirements	979

11.4.2.1	E-UTRA FDD	979

11.4.2.2	E-UTRA TDD	980

11.5	Selection / Reselection of ProSe Synchronization Reference	980

11.5.1	Introduction	980

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	980

11.5.2.1	Introduction	980

11.5.2.2	Requirements	980

11.6	Void	981

11.7	Selection / Reselection of ProSe relay UE	981

11.7.1	Introduction	981

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	981

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	981

12.1	Introduction	981

12.2	Transmit Timing	982

12.2.1	GNSS as timing reference	982

12.3	Interruption	982

12.4	Reliability of GNSS signal	982

13	V2X Requirements	982

13.1	Introduction	982

13.2	UE Transmit Timing	983

13.2.1	Introduction	983

13.2.2	GNSS as synchronization reference source	983

13.2.3	Serving cell/PCell as synchronization reference source	983

13.2.4	SyncRef UE as synchronization reference source	983

13.3	Initiation/Cease of SLSS Transmissions	983

13.3.1	Introduction	983

13.3.1.1	Initiation/Cease of SLSS transmissions with Serving cell / PCell as synchronization reference source	983

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	984

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	984

13.4	Selection / Reselection of V2X Synchronization Reference Source	985

13.5	Autonomous Resource Selection/Reselection measurements	986

13.5.1	Introduction	986

13.5.2	PSSCH-RSRP measurements	986

13.5.3	S-RSSI measurements	986

13.6	Congestion Control measurements	986

13.7	Interruption	986

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	986

13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change	986

13.7.3	Interruptions to WAN due to V2X Carrier Aggregation	987

13.7.4	Interruptions to WAN due to NR V2X sidelink communication	987

13.8	Reliability of GNSS signal	987

13.9	Component Carrier Addition and Release Delay for V2X Sidelink Carrier Aggregation	988

13.10	Selection / Reselection of V2X Synchronization Reference Source for V2X Carrier Aggregation	988

Annex A (normative): Test Cases	997

A.1	Purpose of annex	997

A.2	Requirement classification for statistical testing	997

A.2.1	Types of requirements in TS 36.133	997

A.2.1.1	Time and delay requirements on UE higher layer actions	997

A.2.1.2	Measurements of power levels, relative powers and time	997

A.2.1.3	Implementation requirements	998

A.2.1.4	Physical layer timing requirements	998

A.3	RRM test configurations	999

A.3.1	Reference Measurement Channels	999

A.3.1.1	PDSCH	999

A.3.1.1.1	FDD	999

A.3.1.1.2	TDD	1004

A.3.1.1.3	FDD for UE category 0	1007

A.3.1.1.4	HD-FDD for UE category 0	1008

A.3.1.1.5	TDD for UE category 0	1009

A.3.1.1.6	Frame Structure 3	1010

A.3.1.2	PCFICH/PDCCH/PHICH	1011

A.3.1.2.1	FDD	1011

A.3.1.2.2	TDD	1011

A.3.1.2.3	HD-FDD for UE category 0	1012

A.3.1.2.4	FS 3	1012

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	1012

A.3.1.3.1	FDD in CEModeA	1013

A.3.1.3.2	HD-FDD in CEModeA	1013

A.3.1.3.3	TDD in CEModeA	1014

A.3.1.3.4	FDD in CEModeB	1014

A.3.1.3.5	HD-FDD in CEModeB	1015

A.3.1.3.6	TDD in CEModeB	1015

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	1016

A.3.1.4.1	FDD in CEModeA	1016

A.3.1.4.2	HD-FDD in CEModeA	1017

A.3.1.4.3	TDD in CEModeA	1018

A.3.1.4.4	FDD in CEModeB	1019

A.3.1.4.5	HD-FDD in CEModeB	1020

A.3.1.4.6	TDD in CEModeB	1020

A.3.1.5	NPDSCH Reference Channel for UE category NB1	1021

A.3.1.5.1	HD-FDD in-band operation	1021

A.3.1.5.2	Void	1022

A.3.1.5.3	HD-FDD standalone operation	1022

A.3.1.5.4	Void	1023

A.3.1.5.5	HD-FDD guard band operation	1023

A.3.1.5.6	Void	1024

A.3.1.5.7	TDD in-band operation	1024

A.3.1.5.8	TDD standalone operation	1024

A.3.1.5.9	TDD guard band operation	1025

A.3.1.6	NPDCCH Reference Channel for UE category NB1	1025

A.3.1.6.1	In-band operation	1025

A.3.1.6.2	Void	1026

A.3.1.6.3	Standalone operation	1026

A.3.1.6.4	Void	1026

A.3.1.6.5	Guard band operation	1026

A.3.1.6.6	Void	1027

A.3.2	OFDMA Channel Noise Generator (OCNG)	1027

A.3.2.1	OCNG Patterns for FDD	1027

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	1028

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	1028

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	1029

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	1030

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	1030

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	1031

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	1031

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	1031

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	1032

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	1033

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	1033

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	1034

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	1034

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	1035

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	1035

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	1036

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	1036

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	1037

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	1037

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	1038

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	1038

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	1039

A.3.2.2	OCNG Patterns for TDD	1039

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	1040

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	1040

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	1041

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	1041

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	1042

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	1043

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	1044

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	1045

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	1045

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	1046

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	1047

A.3.2.3	OCNG Patterns for Narrowband IoT	1047

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	1049

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	1050

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	1050

A.3.2.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell	1051

A.3.2.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell	1052

A.3.2.3.6	Narrowband IoT OCNG TDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	1053

A.3.2.3.7	Narrowband IoT OCNG TDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	1054

A.3.2.3.8	Narrowband IoT OCNG TDD pattern 3: standalone NB-IoT	1055

A.3.2.4	OCNG Patterns for V2X sidelink	1056

A.3.2.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	1056

A.3.2.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz	1057

A.3.3	Reference DRX Configurations	1057

A.3.4	ABS Transmission Configurations	1058

A.3.4.1	Non-MBSFN ABS Transmission Configurations	1058

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	1058

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	1058

A.3.4.2	MBSFN ABS Transmission Configurations	1059

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	1059

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	1060

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	1060

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	1060

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	1061

A.3.6.1	Introduction	1061

A.3.7	Test Cases with Different Channel Bandwidths	1061

A.3.7.1	Introduction	1061

A.3.7.2	Principle of testing	1061

A.3.8	Antenna Configuration	1061

A.3.8.1	Antenna connection for 4 Rx capable UEs	1061

A.3.8.1.1 Introduction	1061

A.3.8.1.2 Principle of testing	1061

A.3.8.1.2.1 Single carrier tests	1061

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests	1062

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported	1063

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported	1063

A.3.8.2	Antenna connection for 8 Rx capable UEs	1063

A.3.8.2.1	Introduction	1063

A.3.8.2.2	Principle of testing	1063

A.3.8.2.2.1	Single carrier tests	1063

A.3.8.2.2.2	Carrier aggregation and Dual connectivity tests	1064

A.3.8.2.2.3	Antenna connection for bands where 2RX is supported	1064

A.3.8.2.2.4	Antenna connection for bands where 4RX is supported	1064

A.3.8.2.2.5	Antenna connection for bands where 8RX is supported	1064

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	1064

A.3.9.1	Introduction	1064

A.3.9.2	Principle of testing	1064

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	1065

A.3.10.1	Introduction	1065

A.3.10.2	Principle of testing	1065

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	1065

A.3.11.1	Introduction	1065

A.3.11.2	Principle of Testing	1065

A.3.12	Proximity-based Services	1065

A.3.12.1	Introduction	1065

A.3.12.2	Reference DRX configurations for ProSe tests	1065

A.3.12.3	Test Cases with Different Channel Bandwidths	1066

A.3.12.3.1	Introduction	1066

A.3.12.3.2	Principle of testing	1066

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	1066

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	1073

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	1076

A.3.12.6.1	FDD	1076

A.3.12.7	Reference measurement channels for ProSe Direct Communication	1076

A.3.12.7.1	FDD	1076

A.3.12.8	ProSe Receive Traffic Generator	1077

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	1077

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	1077

A.3.13	Time Offset between Cells	1078

A.3.13.1	Introduction	1078

A.3.13.2	Definition	1078

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	1078

A.3.14.1	Introduction	1078

A.3.14.2	Principle of testing	1078

A.3.15	Dual connectivity test cases with different combination of duplex mode	1078

A.3.15.1	Introduction	1078

A.3.15.2	Principle of testing	1078

A.3.16	Reference PRACH Configurations	1079

A.3.17	Listen before talk model	1079

A.3.17.1	Introduction	1079

A.3.17.2	Definition	1079

A.3.18	Reference NPRACH Configurations	1080

A.3.19	Dual connectivity test cases with different bandwidth combinations	1081

A.3.19.1	Introduction	1081

A.3.19.2	Principle of testing	1081

A.3.20	Category M1 UE Test Cases	1082

A.3.20.1	Introduction	1082

A.3.20.2	Principle of Cat-M1 UE Testing	1082

A.3.20.3	Principle of Cat-M1 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	1083

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	1084

A.3.21.1	Introduction	1084

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	1084

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	1085

A.3.22	Category 1bis UE Test Cases	1086

A.3.22.1	Introduction	1086

A.3.22.2	Principle of Category 1bis UE Testing	1086

A.3.23	Category NB2 UE Test Cases	1090

A.3.23.1	Introduction	1090

A.3.23.2	Principle of Category NB2 UE Testing	1090

A.3.24	V2X sidelink communication	1093

A.3.24.1	Introduction	1093

A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication	1094

A.3.24.3	Reference measurement channels for V2X Sidelink Communication	1098

A.3.25	Category M2 UE Test Cases	1099

A.3.25.1	Introduction	1099

A.3.25.2	Principle of Cat-M2 UE Testing	1099

A.3.25.3	Principle of Cat-M2 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	1100

A.3.26	sTTI and processing time reduction test cases with different sTTI/processing time reduction scheme	1101

A.3.26.1	Introduction	1101

A.3.26.2	Principle of testing	1101

A.3.27	LTE INACTIVE Cell Re-selection Test Cases	1101

A.3.27.1	Introduction	1101

A.3.27.2	Principle of INACTIVE cell re-selection Testing	1101

A.3.28	Testing related to Satellite access	1101

A.3.28.1	Introduction	1101

A.3.28.2	Principle of testing GSO and NGSO scenarios	1101

A.3.28.2	Principle of testing different RRM requirements	1102

A.3.28.3	Principle of testing different ephemeris formats	1106

A.3.28.4	General setup for SIB31/SIB-31-NB	1106

A.3.28.5	Satellite specific parameters configuration	1106

A.3.28.5.1	Satellite specific configuration for serving cell	1106

A.3.28.5.2	Satellite specific configuration for neighbour cell	1106

A.4	E-UTRAN RRC_IDLE state	1107

A.4.2	Cell Re-Selection	1107

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	1107

A.4.2.1.1	Test Purpose and Environment	1107

A.4.2.1.2	Test Requirements	1109

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	1110

A.4.2.2.1	Test Purpose and Environment	1110

A.4.2.2.2	Test Requirements	1111

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	1112

A.4.2.3.1	Test Purpose and Environment	1112

A.4.2.3.2	Test Requirements	1113

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	1114

A.4.2.4.1	Test Purpose and Environment	1114

A.4.2.4.2	Test Requirements	1115

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	1116

A.4.2.5.1	Test Purpose and Environment	1116

A.4.2.5.2	Test Requirements	1117

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	1118

A.4.2.6.1	Test Purpose and Environment	1118

A.4.2.6.2	Test Requirements	1119

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	1120

A.4.2.7.1	Test Purpose and Environment	1120

A.4.2.7.2	Test Requirements	1121

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	1122

A.4.2.8.1	Test Purpose and Environment	1122

A.4.2.8.2	Test Requirements	1123

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	1124

A.4.2.9.1	Test Purpose and Environment	1124

A.4.2.9.2	Test Requirements	1124

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	1124

A.4.2.10.1	Test Purpose and Environment	1124

A.4.2.10.2	Test Requirements	1128

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	1128

A.4.2.11.1	Test Purpose and Environment	1128

A.4.2.11.2	Test Requirements	1132

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	1132

A.4.2.12.1	Test Purpose and Environment	1132

A.4.2.12.2	Test Requirements	1134

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	1135

A.4.2.13.1	Test Purpose and Environment	1135

A.4.2.13.2	Test Requirements	1136

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	1137

A.4.2.14.1	Test Purpose and Environment	1137

A.4.2.14.2	Test Requirements	1138

A.4.2.15 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	1139

A.4.2.15.1	Test Purpose and Environment	1139

A.4.2.15.2	Test Requirements	1140

A.4.2.16 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	1141

A.4.2.16.1	Test Purpose and Environment	1141

A.4.2.16.2	Test Requirements	1142

A.4.2.17 E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	1143

A.4.2.17.1	Test Purpose and Environment	1143

A.4.2.17.2	Test Requirements	1144

A.4.2.18 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	1145

A.4.2.18.1	Test Purpose and Environment	1145

A.4.2.18.2	Test Requirements	1147

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	1148

A.4.2.19.1	Test Purpose and Environment	1148

A.4.2.19.2	Test Requirements	1150

A.4.2.20	E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis	1151

A.4.2.20.1	Test Purpose and Environment	1151

A.4.2.20.2	Test Requirements	1152

A.4.2.21	E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis	1153

A.4.2.21.1	Test Purpose and Environment	1153

A.4.2.21.2	Test Requirements	1154

A.4.2.22	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	1155

A.4.2.22.1	Test Purpose and Environment	1155

A.4.2.22.2	Test Requirements	1156

A.4.2.23	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	1157

A.4.2.23.1	Test Purpose and Environment	1157

A.4.2.23.2	Test Requirements	1158

A.4.2.24	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	1159

A.4.2.24.1	Test Purpose and Environment	1159

A.4.2.24.2	Test Requirements	1162

A.4.2.25	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	1163

A.4.2.25.1	Test Purpose and Environment	1163

A.4.2.25.2	Test Requirements	1164

A.4.2.26	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	1165

A.4.2.26.1	Test Purpose and Environment	1165

A.4.2.26.2	Test Requirements	1166

A.4.2.27	E-UTRAN TDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	1167

A.4.2.27.1	Test Purpose and Environment	1167

A.4.2.27.2	Test Requirements	1168

A.4.2.28	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	1169

A.4.2.28.1	Test Purpose and Environment	1169

A.4.2.28.2	Test Requirements	1170

A.4.2.29	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	1171

A.4.2.29.1	Test Purpose and Environment	1171

A.4.2.29.2	Test Requirements	1172

A.4.2.30	E-UTRAN TDD Inter frequency case for Cat-M1 UE in enhanced coverage	1173

A.4.2.30.1	Test Purpose and Environment	1173

A.4.2.30.2	Test Requirements	1174

A.4.2.31	E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis	1175

A.4.2.31.1	Test Purpose and Environment	1175

A.4.2.31.2	Test Requirements	1176

A.4.2.32	E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis	1177

A.4.2.32.1	Test Purpose and Environment	1177

A.4.2.32.2	Test Requirements	1178

A.4.2.33	E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis	1179

A.4.2.33.1	Test Purpose and Environment	1179

A.4.2.33.2	Test Requirements	1180

A.4.2.34	E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis	1181

A.4.2.34.1	Test Purpose and Environment	1181

A.4.2.34.2	Test Requirements	1182

A.4.2.35	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	1183

A.4.2.35.1	Test Purpose and Environment	1183

A.4.2.35.2	Test Requirements	1185

A.4.2.36	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	1186

A.4.2.36.1	Test Purpose and Environment	1186

A.4.2.36.2	Test Requirements	1188

A.4.2.37	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	1189

A.4.2.37.1	Test Purpose and Environment	1189

A.4.2.37.2	Test Requirements	1191

A.4.2.38	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation	1192

A.4.2.38.1	Test Purpose and Environment	1192

A.4.2.38.2	Test Requirements	1194

A.4.2.39	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1195

A.4.2.39.1	Test Purpose and Environment	1195

A.4.2.39.2	Test Requirements	1196

A.4.2.40	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1197

A.4.2.40.1	Test Purpose and Environment	1197

A.4.2.40.2	Test Requirements	1198

A.4.2.41 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX	1199

A.4.2.41.1	Test Purpose and Environment	1199

A.4.2.41.2	Test Requirements	1202

A.4.2.42	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1203

A.4.2.42.1	Test Purpose and Environment	1203

A.4.2.42.2	Test Requirements	1205

A.4.2.43	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1206

A.4.2.43.1	Test Purpose and Environment	1206

A.4.2.43.2	Test Requirements	1208

A.4.2.44	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX	1209

A.4.2.44.1	Test Purpose and Environment	1209

A.4.2.44.2	Test Requirements	1211

A.4.2.45	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1212

A.4.2.45.1	Test Purpose and Environment	1212

A.4.2.45.2	Test Requirements	1214

A.4.2.46	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1215

A.4.2.46.1	Test Purpose and Environment	1215

A.4.2.46.2	Test Requirements	1217

A.4.2.47	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation with UE specific DRX	1218

A.4.2.47.1	Test Purpose and Environment	1218

A.4.2.47.2	Test Requirements	1220

A.4.2.48	E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1221

A.4.2.48.1	Test Purpose and Environment	1221

A.4.2.48.2	Test Requirements	1222

A.4.2.49	E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1223

A.4.2.49.1	Test Purpose and Environment	1223

A.4.2.49.2	Test Requirements	1225

A.4.2.50	E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1226

A.4.2.50.1	Test Purpose and Environment	1226

A.4.2.50.2	Test Requirements	1228

A.4.2.51 E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1229

A.4.2.51.1	Test Purpose and Environment	1229

A.4.2.51.2	Test Requirements	1231

A.4.2.52 E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1232

A.4.2.52.1	Test Purpose and Environment	1232

A.4.2.52.2	Test Requirements	1234

A.4.2.53 E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1235

A.4.2.53.1	Test Purpose and Environment	1235

A.4.2.53.2	Test Requirements	1236

A.4.2.54	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1237

A.4.2.54.1	Test Purpose and Environment	1237

A.4.2.54.2	Test Requirements	1239

A.4.2.55	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1240

A.4.2.55.1	Test Purpose and Environment	1240

A.4.2.55.2	Test Requirements	1242

A.4.2.56	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	1243

A.4.2.56.1	Test Purpose and Environment	1243

A.4.2.56.2	Test Requirements	1244

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	1245

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	1245

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	1245

A.4.3.1.1.1	Test Purpose and Environment	1245

A.4.3.1.1.2	Test Requirements	1247

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	1247

A.4.3.1.2.1	Test Purpose and Environment	1247

A.4.3.1.2.2	Test Requirements	1250

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	1250

A.4.3.1.3.1	Test Purpose and Environment	1250

A.4.3.1.3.2	Test Requirements	1253

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	1253

A.4.3.1.4.1	Test Purpose and Environment	1253

A.4.3.1.4.2	Test Requirements	1254

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	1254

A.4.3.1.5.1	Test Purpose and Environment	1254

A.4.3.1.5.2	Test Requirements	1257

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	1258

A.4.3.2.1	Test Purpose and Environment	1258

A.4.3.2.1.1	Void	1258

A.4.3.2.1.2	1.28Mcps TDD option	1258

A.4.3.2.1.3	Void	1260

A.4.3.2.2	Test Requirements	1260

A.4.3.2.2.1	1.28Mcps TDD option	1260

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	1260

A.4.3.2A.1	Test Purpose and Environment	1260

A.4.3.2A.2	Test Requirements	1264

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	1264

A.4.3.3.1	Test Purpose and Environment	1264

A.4.3.3.2	Test Requirements	1267

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	1267

A.4.3.3A.1	Test Purpose and Environment	1267

A.4.3.3A.2	Test Requirements	1272

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	1273

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	1273

A.4.3.4.1.1	Test Purpose and Environment	1273

A.4.3.4.1.2	Test Requirements	1275

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	1275

A.4.3.4.2.1	Test Purpose and Environment	1275

A.4.3.4.2.2	Test Requirements	1277

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	1277

A.4.3.4.3.1	Test Purpose and Environment	1277

A.4.3.4.3.2	Test Requirements	1280

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	1280

A.4.3.4.4.1	Test Purpose and Environment	1280

A.4.3.4.4.2	Test Requirements	1284

A.4.4	E-UTRAN to GSM Cell Re-Selection	1284

A.4.4.1	E-UTRAN FDD – GSM:	1284

A.4.4.1.1	Test Purpose and Environment	1284

A.4.4.1.2	Test Requirements	1286

A.4.4.2	E-UTRAN TDD – GSM:	1286

A.4.4.2.1	Test Purpose and Environment	1286

A.4.4.2.2	Test Requirements	1288

A.4.5	E-UTRAN to HRPD Cell Re-Selection	1289

A.4.5.1	E-UTRAN FDD – HRPD	1289

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	1289

A.4.5.1.1.1	Test Purpose and Environment	1289

A.4.5.1.1.2	Test Requirements	1291

A.4.5.2	E-UTRAN TDD – HRPD	1291

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	1291

A.4.5.2.1.1	Test Purpose and Environment	1291

A.4.5.2.1.2	Test Requirements	1294

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	1294

A.4.6.1	E-UTRAN FDD – cdma2000 1X	1294

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1294

A.4.6.1.1.1	Test Purpose and Environment	1294

A.4.6.1.1.2	Test Requirements	1297

A.4.6.2	E-UTRAN TDD – cdma2000 1X	1297

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1297

A.4.6.2.1.1	Test Purpose and Environment	1297

A.4.6.2.1.2	Test Requirements	1300

A.4.7	Idle State Positioning Measurement for UE category NB1	1300

A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1300

A.4.7.1.1	Test Purpose and Environment	1300

A.4.7.1.2	Test Requirements	1305

A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1306

A.4.7.2.1	Test Purpose and Environment	1306

A.4.7.4.2	Test Requirements	1310

A.4.7.3	TDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1311

A.4.7.3.1	Test Purpose and Environment	1311

A.4.7.3.2	Test Requirements	1315

A.4.7.4	TDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1316

A.4.7.4.1	Test Purpose and Environment	1316

A.4.7.4.2	Test Requirements	1320

A.5	E-UTRAN RRC CONNECTED Mode Mobility	1321

A.5.1	E-UTRAN Handover	1321

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	1321

A.5.1.1.1	Test Purpose and Environment	1321

A.5.1.1.2	Test Requirements	1323

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	1323

A.5.1.2.1	Test Purpose and Environment	1323

A.5.1.2.2	Test Requirements	1325

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	1325

A.5.1.3.1	Test Purpose and Environment	1325

A.5.1.3.2	Test Requirements	1327

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	1327

A.5.1.4.1	Test Purpose and Environment	1327

A.5.1.4.2	Test Requirements	1329

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	1329

A.5.1.5.1	Test Purpose and Environment	1329

A.5.1.5.2	Test Requirements	1331

A.5.1.6 	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	1331

A.5.1.6.1	Test Purpose and Environment	1331

A.5.1.6.2	Test Requirements	1333

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	1333

A.5.1.7.1	Test Purpose and Environment	1333

A.5.1.7.2	Test Requirements	1336

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	1336

A.5.1.8.1	Test Purpose and Environment	1336

A.5.1.8.2	Test Requirements	1339

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	1339

A.5.1.9.1	Test Purpose and Environment	1339

A.5.1.9.2	Test Requirements	1340

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	1340

A.5.1.10.1	Test Purpose and Environment	1340

A.5.1.10.2	Test Requirements	1342

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	1342

A.5.1.11.1	Test Purpose and Environment	1342

A.5.1.11.2	Test Requirements	1344

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	1344

A.5.1.12.1	Test Purpose and Environment	1344

A.5.1.12.2	Test Requirements	1346

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1346

A.5.1.13.1	Test Purpose and Environment	1346

A.5.1.13.2	Test Requirements	1348

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1349

A.5.1.14.1	Test Purpose and Environment	1349

A.5.1.14.2	Test Requirements	1350

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	1351

A.5.1.15.1	Test Purpose and Environment	1351

A.5.1.15.2	Test Requirements	1352

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1353

A.5.1.16.1	Test Purpose and Environment	1353

A.5.1.16.2	Test Requirements	1354

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1355

A.5.1.17.1	Test Purpose and Environment	1355

A.5.1.17.2	Test Requirements	1356

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	1357

A.5.1.18.1	Test Purpose and Environment	1357

A.5.1.18.2	Test Requirements	1358

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis	1359

A.5.1.19.1	Test Purpose and Environment	1359

A.5.1.19.2	Test Requirements	1360

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis	1361

A.5.1.20.1	Test Purpose and Environment	1361

A.5.1.20.2	Test Requirements	1362

A.5.1.21	E-UTRAN FDD - FDD Intra frequency RACH-less handover	1363

A.5.1.21.1	Test Purpose and Environment	1363

A.5.1.21.2	Test Requirements	1364

A.5.1.22	E-UTRAN TDD - TDD Intra frequency RACH-less handover	1364

A.5.1.22.1	Test Purpose and Environment	1364

A.5.1.22.2	Test Requirements	1366

A.5.1.23	E-UTRAN FDD – FDD Inter frequency RACH-less handover	1366

A.5.1.23.1	Test Purpose and Environment	1366

A.5.1.23.2	Test Requirements	1368

A.5.1.24	E-UTRAN TDD – TDD Inter frequency RACH-less handover	1368

A.5.1.24.1	Test Purpose and Environment	1368

A.5.1.24.2	Test Requirements	1370

A.5.1.25	E-UTRAN FDD - FDD Intra frequency make-before-break handover	1370

A.5.1.25.1	Test Purpose and Environment	1370

A.5.1.25.2	Test Requirements	1372

A.5.1.26	E-UTRAN TDD - TDD Intra frequency make-before-break handover	1373

A.5.1.26.1	Test Purpose and Environment	1373

A.5.1.26.2	Test Requirements	1374

A.5.1.27	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA	1375

A.5.1.27.1	Test Purpose and Environment	1375

A.5.1.27.2	Test Requirements	1376

A.5.1.28	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA	1377

A.5.1.28.1	Test Purpose and Environment	1377

A.5.1.28.2	Test Requirements	1378

A.5.1.29	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeA	1379

A.5.1.29.1	Test Purpose and Environment	1379

A.5.1.29.2	Test Requirements	1380

A.5.1.30	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB	1381

A.5.1.30.1	Test Purpose and Environment	1381

A.5.1.30.2	Test Requirements	1382

A.5.1.31	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB	1383

A.5.1.31.1	Test Purpose and Environment	1383

A.5.1.31.2	Test Requirements	1384

A.5.1.32	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeB	1385

A.5.1.32.1	Test Purpose and Environment	1385

A.5.1.32.2	Test Requirements	1386

A.5.1.33	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1387

A.5.1.33.1	Test Purpose and Environment	1387

A.5.1.13.2	Test Requirements	1388

A.5.1.34	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1389

A.5.1.34.1	Test Purpose and Environment	1389

A.5.1.34.2	Test Requirements	1390

A.5.1.35	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1391

A.5.1.35.1	Test Purpose and Environment	1391

A.5.1.35.2	Test Requirements	1392

A.5.1.36	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1393

A.5.1.36.1	Test Purpose and Environment	1393

A.5.1.36.2	Test Requirements	1394

A.5.1.37	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1395

A.5.1.37.1	Test Purpose and Environment	1395

A.5.1.37.2	Test Requirements	1396

A.5.1.38	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1397

A.5.1.38.1	Test Purpose and Environment	1397

A.5.1.38.2	Test Requirements	1398

A.5.1.39	E-UTRAN FDD - FDD Intra frequency handover with direct SCell activation	1399

A.5.1.39.1	Test Purpose and Environment	1399

A.5.1.39.2	Test Requirements	1400

A.5.1.40	E-UTRAN TDD - TDD Intra frequency handover with direct SCell activation	1401

A.5.1.40.1	Test Purpose and Environment	1401

A.5.1.40.2	Test Requirements	1402

A.5.1.41	E-UTRAN FDD – FDD Intra-band Inter-frequency sync DAPS handover	1403

A.5.1.41.1	Test Purpose and Environment	1403

A.5.1.41.2	Test Requirements	1405

A.5.1.42	E-UTRAN FDD – FDD Intra-band Inter-frequency async DAPS handover	1406

A.5.1.42.1	Test Purpose and Environment	1406

A.5.1.42.2	Test Requirements	1407

A.5.1.43	E-UTRAN FDD – FDD Inter-band Inter-frequency sync DAPS handover	1408

A.5.1.43.1	Test Purpose and Environment	1408

A.5.1.43.2	Test Requirements	1409

A.5.1.44	E-UTRAN FDD – FDD Inter-band Inter-frequency async DAPS handover	1410

A.5.1.44.1	Test Purpose and Environment	1410

A.5.1.44.2	Test Requirements	1411

A.5.1.45	E-UTRAN FDD - FDD Intra frequency DAPS handover	1412

A.5.1.45.1	Test Purpose and Environment	1412

A.5.1.45.1	Test Requirements	1413

A.5.1.46	E-UTRAN TDD - TDD Intra frequency DAPS handover	1414

A.5.1.46.1	Test Purpose and Environment	1414

A.5.1.46.2	Test Requirements	1415

A.5.1.47	E-UTRAN FDD - FDD Intra frequency conditional handover	1416

A.5.1.47.1	Test Purpose and Environment	1416

A.5.1.47.2	Test Requirements	1417

A.5.1.48	E-UTRAN TDD - TDD Intra frequency conditional handover	1418

A.5.1.48.1	Test Purpose and Environment	1418

A.5.1.48.2	Test Requirements	1419

A.5.1.49	E-UTRAN FDD - FDD Inter frequency conditional handover	1420

A.5.1.49.1	Test Purpose and Environment	1420

A.5.1.49.2	Test Requirements	1421

A.5.1.50	E-UTRAN TDD - TDD Inter frequency conditional handover	1422

A.5.1.50.1	Test Purpose and Environment	1422

A.5.1.50.2	Test Requirements	1423

A.5.1.51	E-UTRAN FDD - TDD Inter frequency conditional handover	1424

A.5.1.51.1	Test Purpose and Environment	1424

A.5.1.51.2	Test Requirements	1426

A.5.1.52	E-UTRAN TDD - FDD Inter frequency conditional handover	1426

A.5.152.1	Test Purpose and Environment	1426

A.5.1.52.2	Test Requirements	1429

A.5.1.53	E-UTRAN TDD – TDD Intra-band Inter-frequency sync DAPS handover	1430

A.5.1.53.1	Test Purpose and Environment	1430

A.5.1.53.2	Test Requirements	1432

A.5.1.54	E-UTRAN TDD – TDD Inter-band Inter-frequency sync DAPS handover	1433

A.5.1.54.1	Test Purpose and Environment	1433

A.5.1.54.2	Test Requirements	1435

A.5.1.55	E-UTRAN FDD - TDD inter-band inter-frequency synchronous DAPS handover	1436

A.5.1.55.1	Test Purpose and Environment	1436

A.5.1.56	E-UTRAN TDD - FDD inter-band inter-frequency synchronous DAPS handover	1440

A.5.1.56.1	Test Purpose and Environment	1440

A.5.1.56.2	Test Requirements	1443

A.5.1.57	E-UTRAN FDD – TDD Inter-band Inter-frequency async DAPS handover	1444

A.5.1.57.1	Test Purpose and Environment	1444

A.5.1.57.2	Test Requirements	1446

A.5.1.58	E-UTRAN TDD – FDD Inter-band Inter-frequency async DAPS handover	1446

A.5.1.58.1	Test Purpose and Environment	1446

A.5.1.58.2	Test Requirements	1449

A.5.2	E-UTRAN Handover to other RATs	1449

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	1449

A.5.2.1.1	Test Purpose and Environment	1449

A.5.2.1.2	Test Requirements	1451

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	1452

A.5.2.2.1	Test Purpose and Environment	1452

A.5.2.2.2	Test Requirements	1455

A.5.2.3 E-UTRAN FDD- GSM Handover	1455

A.5.2.3.1	Test Purpose and Environment	1455

A.5.2.3.2	Test Requirements	1456

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	1457

A.5.2.4.1	Test Purpose and Environment	1457

A.5.2.4.1.1	Void	1457

A.5.2.4.1.2	1.28 Mcps TDD option	1457

A.5.2.4.1.3	Void	1459

A.5.2.4.2	Test Requirements	1459

A.5.2.4.2.1	Void	1459

A.5.2.4.2.2	1.28 Mcps TDD option	1459

A.5.2.4.2.3	Void	1459

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	1459

A.5.2.5.1	Test Purpose and Environment	1459

A.5.2.5.1.3	Void	1462

A.5.2.5.2	Test Requirements	1462

A.5.2.5.2.1	Void	1462

A.5.2.5.2.2	1.28 Mcps TDD option	1462

A.5.2.5.2.3	Void	1462

A.5.2.6	E-UTRAN TDD - GSM Handover	1462

A.5.2.6.1	Test Purpose and Environment	1462

A.5.2.6.2	Test Requirements	1464

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	1465

A.5.2.7.1	Test Purpose and Environment	1465

A.5.2.7.2	Test Requirements	1467

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	1467

A.5.2.8.1	Test Purpose and Environment	1467

A.5.2.8.2	Test Requirements	1468

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	1469

A.5.2.9.1	Test Purpose and Environment	1469

A.5.2.9.2	Test Requirements	1470

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	1471

A.5.2.10.1	Test Purpose and Environment	1471

A.5.2.10.2	Test Requirements	1473

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	1473

A.5.2.10A.1	Test Purpose and Environment	1473

A.5.2.10A.2	Test Requirements	1476

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	1476

A.5.2.10B.1	Test Purpose and Environment	1476

A.5.2.10B.2	Test Requirements	1479

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	1479

A.5.2.11.1	Test Purpose and Environment	1479

A.5.2.11.2	Test Requirements	1479

A.5.3	E-UTRAN Handover to Non-3GPP RATs	1480

A.5.3.1	E-UTRAN FDD – HRPD Handover	1480

A.5.3.1.1	Test Purpose and Environment	1480

A.5.3.1.2	Test Requirements	1482

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	1482

A.5.3.2.1	Test Purpose and Environment	1482

A.5.3.2.2	Test Requirements	1485

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	1485

A.5.3.3.1	Test Purpose and Environment	1485

A.5.3.3.2	Test Requirements	1488

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	1488

A.5.3.4.1	Test Purpose and Environment	1488

A.5.3.4.2	Test Requirements	1490

A.5.3.5	E-UTRAN TDD – HRPD Handover	1490

A.5.3.5.1	Test Purpose and Environment	1490

A.5.3.5.2	Test Requirements	1493

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	1493

A.5.3.6.1	Test Purpose and Environment	1493

A.5.3.6.2	Test Requirements	1496

A.6	RRC Connection Control	1496

A.6.1	RRC Re-establishment	1496

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	1496

A.6.1.1.1	Test Purpose and Environment	1496

A.6.1.1.2	Test Requirements	1497

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	1498

A.6.1.2.1	Test Purpose and Environment	1498

A.6.1.2.2	Test Requirements	1499

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	1500

A.6.1.3.1	Test Purpose and Environment	1500

A.6.1.3.2	Test Requirements	1501

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	1502

A.6.1.4.1	Test Purpose and Environment	1502

A.6.1.4.2	Test Requirements	1503

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	1504

A.6.1.5.1	Test Purpose and Environment	1504

A.6.1.5.2	Test Requirements	1504

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	1504

A.6.1.6.1	Test Purpose and Environment	1504

A.6.1.6.2	Test Requirements	1506

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	1507

A.6.1.7.1	Test Purpose and Environment	1507

A.6.1.7.2	Test Requirements	1508

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	1509

A.6.1.8.1	Test Purpose and Environment	1509

A.6.1.8.2	Test Requirements	1510

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1511

A.6.1.9.1	Test Purpose and Environment	1511

A.6.1.9.2	Test Requirements	1512

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1513

A.6.1.10.1	Test Purpose and Environment	1513

A.6.1.10.2	Test Requirements	1514

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1515

A.6.1.11.1	Test Purpose and Environment	1515

A.6.1.11.2	Test Requirements	1516

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1517

A.6.1.12.1	Test Purpose and Environment	1517

A.6.1.12.2	Test Requirements	1518

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1519

A.6.1.13.1	Test Purpose and Environment	1519

A.6.1.13.2	Test Requirements	1520

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1521

A.6.1.14.1	Test Purpose and Environment	1521

A.6.1.14.2	Test Requirements	1522

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1523

A.6.1.15.1	Test Purpose and Environment	1523

A.6.1.15.2	Test Requirements	1525

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1526

A.6.1.16.1	Test Purpose and Environment	1526

A.6.1.16.2	Test Requirements	1528

A.6.1.17	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1529

A.6.1.17.1	Test Purpose and Environment	1529

A.6.1.17.2	Test Requirements	1530

A.6.1.18	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1531

A.6.1.18.1	Test Purpose and Environment	1531

A.6.1.18.2	Test Requirements	1532

A.6.1.19	E-UTRAN TDD-TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1533

A.6.1.19.1	Test Purpose and Environment	1533

A.6.1.19.2	Test Requirements	1534

A.6.1.20	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1535

A.6.1.20.1	Test Purpose and Environment	1535

A.6.1.20.2	Test Requirements	1536

A.6.1.21	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1537

A.6.1.21.1	Test Purpose and Environment	1537

A.6.1.21.2	Test Requirements	1538

A.6.1.22	E-UTRAN TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1539

A.6.1.22.1	Test Purpose and Environment	1539

A.6.1.22.2	Test Requirements	1540

A.6.1.23	E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1541

A.6.1.23.1	Test Purpose and Environment	1541

A.6.1.23.2	Test Requirements	1543

A.6.1.24	E-UTRAN TDD - TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1544

A.6.1.24.1	Test Purpose and Environment	1544

A.6.1.24.2	Test Requirements	1546

A.6.2	Random Access	1547

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	1547

A.6.2.1.1	Test Purpose and Environment	1547

A.6.2.1.2	Test Requirements	1548

A.6.2.1.2.1	Random Access Response Reception	1548

A.6.2.1.2.2	No Random Access Response Reception	1548

A.6.2.1.2.3	Receiving a NACK on msg3	1548

A.6.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1549

A.6.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI	1549

A.6.2.1.2.6	Contention Resolution Timer expiry	1549

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	1549

A.6.2.2.1	Test Purpose and Environment	1549

A.6.2.2.2	Test Requirements	1550

A.6.2.2.2.1	Random Access Response Reception	1551

A.6.2.2.2.2	No Random Access Response Reception	1551

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	1551

A.6.2.3.1	Test Purpose and Environment	1551

A.6.2.3.2	Test Requirements	1553

A.6.2.3.2.1	Random Access Response Reception	1553

A.6.2.3.2.2	No Random Access Response reception	1553

A.6.2.3.2.3	Receiving a NACK on msg3	1553

A.6.2.3.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1554

A.6.2.3.2.5	Reception of a Correct Message over Temporary C-RNTI	1554

A.6.2.3.2.6	Contention Resolution Timer expiry	1554

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	1554

A.6.2.4.1	Test Purpose and Environment	1554

A.6.2.4.2	Test Requirements	1556

A.6.2.4.2.1	Random Access Response Reception	1556

A.6.2.4.2.2	No Random Access Response Reception	1556

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	1556

A.6.2.5.1	Test Purpose and Environment	1556

A.6.2.5.2	Test Requirements	1557

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	1557

A.6.2.6.1	Test Purpose and Environment	1557

A.6.2.6.2	Test Requirements	1557

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	1558

A.6.2.7.1	Test Purpose and Environment	1558

A.6.2.7.2	Test Requirements	1560

A.6.2.7.2.1	Random Access Response Reception	1560

A.6.2.7.2.2	No Random Access Response Reception	1560

A.6.2.7.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1560

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	1561

A.6.2.8.1	Test Purpose and Environment	1561

A.6.2.8.2	Test Requirements	1563

A.6.2.8.2.1	Random Access Response Reception	1563

A.6.2.8.2.2	No Random Access Response Reception	1563

A.6.2.8.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1563

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	1564

A.6.2.9.1	Test Purpose and Environment	1564

A.6.2.9.2	Test Requirements	1567

A.6.2.9.2.1	Random Access Response Reception	1567

A.6.2.9.2.2	No Random Access Response Reception	1568

A.6.2.9.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1568

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1569

A.6.2.10.1	Test Purpose and Environment	1569

A.6.2.10.2	Test Requirements	1571

A.6.2.10.2.1	Random Access Response Reception	1571

A.6.2.10.2.2	No Random Access Response Reception	1572

A.6.2.10.2.3	Receiving a NACK on msg3	1572

A.6.2.10.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1572

A.6.2.10.2.5	Reception of a Correct Message over Temporary C-RNTI	1572

A.6.2.10.2.6	Contention Resolution Timer expiry	1572

A.6.2.10.2.7	PRACH Resource Selection	1573

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1573

A.6.2.11.1	Test Purpose and Environment	1573

A.6.2.11.2	Test Requirements	1575

A.6.2.11.2.1	Random Access Response Reception	1575

A.6.2.11.2.2	No Random Access Response Reception	1576

A.6.2.11.2.3	Receiving a NACK on msg3	1576

A.6.2.11.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1576

A.6.2.11.2.5	Reception of a Correct Message over Temporary C-RNTI	1576

A.6.2.11.2.6	Contention Resolution Timer expiry	1576

A.6.2.11.2.7	PRACH Resource Selection	1577

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1577

A.6.2.12.1	Test Purpose and Environment	1577

A.6.2.12.2	Test Requirements	1579

A.6.2.12.2.1	Random Access Response Reception	1579

A.6.2.12.2.2	No Random Access Response Reception	1580

A.6.2.12.2.3	Receiving a NACK on msg3	1580

A.6.2.12.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1580

A.6.2.12.2.5	Reception of a Correct Message over Temporary C-RNTI	1580

A.6.2.12.2.6	Contention Resolution Timer expiry	1580

A.6.2.12.2.7	PRACH Resource Selection	1581

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1581

A.6.2.13.1	Test Purpose and Environment	1581

A.6.2.13.2	Test Requirements	1583

A.6.2.13.2.1	Random Access Response Reception	1583

A.6.2.13.2.2	No Random Access Response Reception	1584

A.6.2.13.2.3	Receiving a NACK on msg3	1584

A.6.2.13.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1584

A.6.2.13.2.5	Reception of a Correct Message over Temporary C-RNTI	1584

A.6.2.13.2.6	Contention Resolution Timer expiry	1584

A.6.2.13.2.7	PRACH Resource Selection	1585

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1585

A.6.2.14.1	Test Purpose and Environment	1585

A.6.2.14.2	Test Requirements	1587

A.6.2.14.2.1	Random Access Response Reception	1587

A.6.2.14.2.2	No Random Access Response Reception	1588

A.6.2.14.2.3	Receiving a NACK on msg3	1588

A.6.2.14.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1588

A.6.2.14.2.5	Reception of a Correct Message over Temporary C-RNTI	1588

A.6.2.14.2.6	Contention Resolution Timer expiry	1588

A.6.2.14.2.7	PRACH Resource Selection	1589

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1589

A.6.2.15.1	Test Purpose and Environment	1589

A.6.2.15.2	Test Requirements	1591

A.6.2.15.2.1	Random Access Response Reception	1591

A.6.2.15.2.2	No Random Access Response Reception	1592

A.6.2.15.2.3	Receiving a NACK on msg3	1592

A.6.2.15.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1592

A.6.2.15.2.5	Reception of a Correct Message over Temporary C-RNTI	1592

A.6.2.15.2.6	Contention Resolution Timer expiry	1592

A.6.2.15.2.7	PRACH Resource Selection	1593

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1593

A.6.2.16.1	Test Purpose and Environment	1593

A.6.2.16.2	Test Requirements	1596

A.6.2.16.2.1	Random Access Response Reception	1596

A.6.2.16.2.2	No Random Access Response Reception	1597

A.6.2.16.2.3	Receiving a NACK on msg3	1597

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1597

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI	1597

A.6.2.16.2.6	Contention Resolution Timer expiry	1597

A.6.2.16.2.7	NPRACH Resource Selection	1597

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1598

A.6.2.17.1	Test Purpose and Environment	1598

A.6.2.17.2	Test Requirements	1601

A.6.2.17.2.1	Random Access Response Reception	1601

A.6.2.17.2.2	No Random Access Response Reception	1602

A.6.2.17.2.3	Receiving a NACK on msg3	1602

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1602

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI	1602

A.6.2.17.2.6	Contention Resolution Timer expiry	1602

A.6.2.17.2.7	NPRACH Resource Selection	1602

A.6.2.18	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1603

A.6.2.18.1	Test Purpose and Environment	1603

A.6.2.18.2	Test Requirements	1606

A.6.2.18.2.1	Random Access Response Reception	1606

A.6.2.18.2.2	No Random Access Response Reception	1607

A.6.2.18.2.3	Receiving a NACK on msg3	1607

A.6.2.18.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1607

A.6.2.18.2.5	Reception of a Correct Message over Temporary C-RNTI	1607

A.6.2.18.2.6	Contention Resolution Timer expiry	1607

A.6.2.18.2.7	NPRACH Resource Selection	1607

A.6.2.19	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1608

A.6.2.19.1	Test Purpose and Environment	1608

A.6.2.19.2	Test Requirements	1610

A.6.2.19.2.1	Random Access Response Reception	1610

A.6.2.19.2.2	No Random Access Response Reception	1611

A.6.2.19.2.3	Receiving a NACK on msg3	1611

A.6.2.19.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1611

A.6.2.19.2.5	Reception of a Correct Message over Temporary C-RNTI	1611

A.6.2.19.2.6	Contention Resolution Timer expiry	1611

A.6.2.19.2.7	NPRACH Resource Selection	1611

A.6.2.20	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in enhanced coverage	1612

A.6.2.20.1	Test Purpose and Environment	1612

A.6.2.20.2	Test Requirements	1614

A.6.2.20.2.1	Random Access Response Reception	1614

A.6.2.20.2.2	No Random Access Response Reception	1615

A.6.2.20.2.3	Receiving a NACK on msg3	1615

A.6.2.20.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1615

A.6.2.20.2.5	Reception of a Correct Message over Temporary C-RNTI	1615

A.6.2.20.2.6	Contention Resolution Timer expiry	1615

A.6.2.20.2.7	NPRACH Resource Selection	1615

A.6.2.21	TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1616

A.6.2.21.1	Test Purpose and Environment	1616

A.6.2.21.2	Test Requirements	1618

A.6.2.21.2.1	Random Access Response Reception	1618

A.6.2.21.2.2	No Random Access Response Reception	1619

A.6.2.21.2.3	Receiving a NACK on msg3	1619

A.6.2.21.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1619

A.6.2.21.2.5	Reception of a Correct Message over Temporary C-RNTI	1619

A.6.2.21.2.6	Contention Resolution Timer expiry	1619

A.6.2.21.2.7	NPRACH Resource Selection	1619

A.6.3	RRC Connection Release with Redirection	1620

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	1620

A.6.3.1.1	Test Purpose and Environment	1620

A.6.3.1.2	Test Requirements	1622

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	1622

A.6.3.2.1	Test Purpose and Environment	1622

A.6.3.2.2	Test Requirements	1624

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	1624

A.6.3.3.1	Test Purpose and Environment	1624

A.6.3.3.2	Test Requirements	1625

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	1626

A.6.3.4.1	Test Purpose and Environment	1626

A.6.3.4.2	Test Requirements	1627

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	1628

A.6.3.5.1	Test Purpose and Environment	1628

A.6.3.5.2	Test Requirements	1630

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	1630

A.6.3.6.1	Test Purpose and Environment	1630

A. 6.3.6.2	Test Requirements	1633

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	1633

A.6.3.7.1	Test Purpose and Environment	1633

A.6.3.7.2	Test Requirements	1636

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	1636

A.6.3.8.1	Test Purpose and Environment	1636

A.6.3.8.2	Test Requirements	1639

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	1639

A.6.3.9.1	Test Purpose and Environment	1639

A.6.3.9.2	Test Requirements	1641

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	1641

A.6.3.10.1	Test Purpose and Environment	1641

A.6.3.10.2	Test Requirements	1643

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	1643

A.6.3.11.1	Test Purpose and Environment	1643

A.6.3.11.2	Test Requirements	1645

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	1645

A.6.3.12.1	Test Purpose and Environment	1645

A.6.3.12.2	Test Requirements	1648

A.6.3.13	Redirection from E-UTRA to NR FR1 for redcap UE	1648

A.6.3.13.1	Test Purpose and Environment	1648

A.6.3.13.2	Test Parameters	1648

A.6.3.13.3	Test Requirements	1652

A.7	Timing and Signalling Characteristics	1653

A.7.1	UE Transmit Timing	1653

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	1653

A.7.1.1.1	Test Purpose and Environment	1653

A.7.1.1.2	Test Requirements	1654

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	1655

A.7.1.2.1	Test Purpose and Environment	1655

A.7.1.2.2	Test Requirements	1657

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	1658

A.7.1.3.1	Test Purpose and Environment	1658

A.7.1.3.2	Test Requirements	1660

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	1661

A.7.1.4.1	Test Purpose and Environment	1661

A.7.1.4.2	Test Requirements	1662

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	1663

A.7.1.4A.1	Test Purpose and Environment	1663

A.7.1.4A.2	Test Requirements	1663

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	1663

A.7.1.5.1	Test Purpose and Environment	1663

A.7.1.5.2	Test Requirements	1664

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	1664

A.7.1.6.1	Test Purpose and Environment	1664

A.7.1.6.2	Test Requirements	1666

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	1667

A.7.1.7.1	Test Purpose and Environment	1667

A.7.1.7.2	Test Requirements	1668

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	1669

A.7.1.7A.1	Test Purpose and Environment	1669

A.7.1.7A.2	Test Requirements	1669

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	1669

A.7.1.7B.1	Test Purpose and Environment	1669

A.7.1.7B.2	Test Requirements	1669

A.7.1.8	Void	1670

A.7.1.8.1	Void	1670

A.7.1.8.2	Void	1670

A.7.1.9	Void	1670

A.7.1.9.1	Void	1670

A.7.1.9.2	Void	1670

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1670

A.7.1.10.1	Test Purpose and Environment	1670

A.7.1.10.2	Test Requirements	1672

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1672

A.7.1.11.1	Test Purpose and Environment	1672

A.7.1.11.2	Test Requirements	1674

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1674

A.7.1.12.1	Test Purpose and Environment	1674

A.7.1.12.2	Test Requirements	1676

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	1676

A.7.1.13.1	Test Purpose and Environment	1676

A.7.1.13.2	Test Requirements	1678

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1679

A.7.1.14.1	Test Purpose and Environment	1679

A.7.1.14.2	Test Requirements	1680

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1680

A.7.1.15.1	Test Purpose and Environment	1680

A.7.1.15.2	Test Requirements	1681

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1682

A.7.1.16.1	Test Purpose and Environment	1682

A.7.1.16.2	Test Requirements	1683

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1684

A.7.1.17.1	Test Purpose and Environment	1684

A.7.1.17.2	Test Requirements	1686

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1687

A.7.1.18.1	Test Purpose and Environment	1687

A.7.1.18.2	Test Requirements	1689

A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1690

A.7.1.19.1	Test Purpose and Environment	1690

A.7.1.19.2	Test Requirements	1691

A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1691

A.7.1.20.1	Test Purpose and Environment	1691

A.7.1.20.2	Test Requirements	1693

A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1693

A.7.1.21.1	Test Purpose and Environment	1693

A.7.1.21.2	Test Requirements	1695

A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1695

A.7.1.22.1	Test Purpose and Environment	1695

A.7.1.22.2	Test Requirements	1697

A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1697

A.7.1.23.1	Test Purpose and Environment	1697

A.7.1.23.2	Test Requirements	1699

A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1699

A.7.1.24.1	Test Purpose and Environment	1699

A.7.1.24.2	Test Requirements	1700

A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1701

A.7.1.25.1	Test Purpose and Environment	1701

A.7.1.25.2	Test Requirements	1702

A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1703

A.7.1.26.1	Test Purpose and Environment	1703

A.7.1.26.2	Test Requirements	1704

A.7.1.27	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1705

A.7.1.27.1	Test Purpose and Environment	1705

A.7.1.27.2	Test Requirements	1707

A.7.1.28	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1707

A.7.1.28.1	Test Purpose and Environment	1707

A.7.1.28.2	Test Requirements	1709

A.7.2	UE Timing Advance	1710

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	1710

A.7.2.1.1	Test Purpose and Environment	1710

A.7.2.1.2	Test Requirements	1711

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	1712

A.7.2.2.1	Test Purpose and Environment	1712

A.7.2.2.2	Test Requirements	1714

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	1714

A.7.2.3.1	Test Purpose and Environment	1714

A.7.2.3.2	Test Requirements	1714

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	1714

A.7.2.4.1	Test Purpose and Environment	1714

A.7.2.4.2	Test Requirements	1717

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	1717

A.7.2.5.1	Test Purpose and Environment	1717

A.7.2.5.2	Test Requirements	1719

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	1719

A.7.2.5A.1	Test Purpose and Environment	1719

A.7.2.5A.2	Test Requirements	1720

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	1720

A.7.2.5B.1	Test Purpose and Environment	1720

A.7.2.5B.2	Test Requirements	1720

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1720

A.7.2.6.1	Test Purpose and Environment	1720

A.7.2.6.2	Test Requirements	1723

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1723

A.7.2.7.1	Test Purpose and Environment	1723

A.7.2.7.2	Test Requirements	1725

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1725

A.7.2.8.1	Test Purpose and Environment	1725

A.7.2.8.2	Test Requirements	1727

A.7.2.9.2	Test Requirements	1728

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1729

A.7.2.10.1	Test Purpose and Environment	1729

A.7.2.10.2	Test Requirements	1730

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1730

A.7.2.11.1	Test Purpose and Environment	1730

A.7.2.11.2	Test Requirements	1732

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1732

A.7.2.12.1	Test Purpose and Environment	1732

A.7.2.12.2	Test Requirements	1734

A.7.2.13	E-UTRAN FDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1735

A.7.2.13.1	Test Purpose and Environment	1735

A.7.2.13.2	Test Requirements	1736

A.7.2.14	E-UTRAN TDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1737

A.7.2.14.1	Test Purpose and Environment	1737

A.7.2.14.2	Test Requirements	1739

A.7.2.15	E-UTRAN TDD – TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage	1739

A.7.2.15.1	Test Purpose and Environment	1739

A.7.2.15.2	Test Requirements	1741

A.7.3	Radio Link Monitoring	1741

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1741

A.7.3.1.1	Test Purpose and Environment	1741

A.7.3.1.2	Test Requirements	1745

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1745

A.7.3.2.1	Test Purpose and Environment	1745

A.7.3.2.2	Test Requirements	1749

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1749

A.7.3.3.1	Test Purpose and Environment	1749

A.7.3.3.2	Test Requirements	1753

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1753

A.7.3.4.1	Test Purpose and Environment	1753

A.7.3.4.2	Test Requirements	1757

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1757

A.7.3.5.1	Test Purpose and Environment	1757

A.7.3.5.2	Test Requirements	1760

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1761

A.7.3.6.1	Test Purpose and Environment	1761

A.7.3.6.2	Test Requirements	1764

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1764

A.7.3.7.1	Test Purpose and Environment	1764

A.7.3.7.2	Test Requirements	1767

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1767

A.7.3.8.1	Test Purpose and Environment	1767

A.7.3.8.2	Test Requirements	1770

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1770

A.7.3.9.2	Test Requirements	1773

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1773

A.7.3.10.1	Test Purpose and Environment	1773

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1777

A.7.3.11.1	Test Purpose and Environment	1777

A.7.3.11.2	Test Requirements	1782

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1782

A.7.3.12.1	Test Purpose and Environment	1782

A.7.3.12.2	Test Requirements	1787

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1787

A.7.3.13.1	Test Purpose and Environment	1787

A.7.3.13.2	Test Requirements	1790

A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1790

A.7.3.14.1	Test Purpose and Environment	1790

A.7.3.14.2	Test Requirements	1793

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1793

A.7.3.15.1	Test Purpose and Environment	1793

A.7.3.15.2	Test Requirements	1797

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1797

A.7.3.16.1	Test Purpose and Environment	1797

A.7.3.16.2	Test Requirements	1802

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1802

A.7.3.17.1	Test Purpose and Environment	1802

A.7.3.17.2	Test Requirements	1806

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1806

A.7.3.18.1	Test Purpose and Environment	1806

A.7.3.18.2	Test Requirements	1810

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1810

A.7.3.19.1	Test Purpose and Environment	1810

A.7.3.19.2	Test Requirements	1815

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1815

A.7.3.20.1	Test Purpose and Environment	1815

A.7.3.20.2	Test Requirements	1820

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1820

A.7.3.21.1	Test Purpose and Environment	1820

A.7.3.21.2	Test Requirements	1825

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1825

A.7.3.22.1	Test Purpose and Environment	1825

A.7.3.22.2	Test Requirements	1830

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1830

A.7.3.23.1	Test Purpose and Environment	1830

A.7.3.23.2	Test Requirements	1831

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1831

A.7.3.24.1	Test Purpose and Environment	1831

A.7.3.24.2	Test Requirements	1832

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1832

A.7.3.25.1	Test Purpose and Environment	1832

A.7.3.25.2	Test Requirements	1833

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1833

A.7.3.26.1	Test Purpose and Environment	1833

A.7.3.26.2	Test Requirements	1836

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1836

A.7.3.27.1	Test Purpose and Environment	1836

A.7.3.27.2	Test Requirements	1839

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1839

A.7.3.28.1	Test Purpose and Environment	1839

A.7.3.28.2	Test Requirements	1842

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1842

A.7.3.29.1	Test Purpose and Environment	1842

A.7.3.29.2	Test Requirements	1845

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1845

A.7.3.30.1	Test Purpose and Environment	1845

A.7.3.30.2	Test Requirements	1848

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1848

A.7.3.31.1	Test Purpose and Environment	1848

A.7.3.31.2	Test Requirements	1851

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1851

A.7.3.32.1	Test Purpose and Environment	1851

A.7.3.32.2	Test Requirements	1854

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1854

A.7.3.33.1	Test Purpose and Environment	1854

A.7.3.33.2	Test Requirements	1857

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1857

A.7.3.34.1	Test Purpose and Environment	1857

A.7.3.34.2	Test Requirements	1860

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1860

A.7.3.35.1	Test Purpose and Environment	1860

A.7.3.35.2	Test Requirements	1863

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1863

A.7.3.36.1	Test Purpose and Environment	1863

A.7.3.36.2	Test Requirements	1866

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1866

A.7.3.37.1	Test Purpose and Environment	1866

A.7.3.37.2	Test Requirements	1869

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1869

A.7.3.38.1	Test Purpose and Environment	1869

A.7.3.38.2	Test Requirements	1872

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1872

A.7.3.39.1	Test Purpose and Environment	1872

A.7.3.39.2	Test Requirements	1875

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1876

A.7.3.40.1	Test Purpose and Environment	1876

A.7.3.40.2	Test Requirements	1878

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1879

A.7.3.41.1	Test Purpose and Environment	1879

A.7.3.41.2	Test Requirements	1882

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1882

A.7.3.42.1	Test Purpose and Environment	1882

A.7.3.42.2	Test Requirements	1885

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1885

A.7.3.43.1	Test Purpose and Environment	1885

A.7.3.43.2	Test Requirements	1888

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1888

A.7.3.44.1	Test Purpose and Environment	1888

A.7.3.44.2	Test Requirements	1891

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1892

A.7.3.45.1	Test Purpose and Environment	1892

A.7.3.45.2	Test Requirements	1894

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1895

A.7.3.46.1	Test Purpose and Environment	1895

A.7.3.46.2	Test Requirements	1898

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1898

A.7.3.47.1	Test Purpose and Environment	1898

A.7.3.47.2	Test Requirements	1901

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1901

A.7.3.48.1	Test Purpose and Environment	1901

A.7.3.48.2	Test Requirements	1904

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1904

A.7.3.49.1	Test Purpose and Environment	1904

A.7.3.49.2	Test Requirements	1907

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1907

A.7.3.50.1	Test Purpose and Environment	1907

A.7.3.50.2	Test Requirements	1910

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1910

A.7.3.51.1	Test Purpose and Environment	1910

A.7.3.51.2	Test Requirements	1913

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1913

A.7.3.52.1	Test Purpose and Environment	1913

A.7.3.52.2	Test Requirements	1916

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1916

A.7.3.53.1	Test Purpose and Environment	1916

A.7.3.53.2	Test Requirements	1919

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1919

A.7.3.54.1	Test Purpose and Environment	1919

A.7.3.54.2	Test Requirements	1922

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1922

A.7.3.55.1	Test Purpose and Environment	1922

A.7.3.55.2	Test Requirements	1925

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1925

A.7.3.56.1	Test Purpose and Environment	1925

A.7.3.56.2	Test Requirements	1928

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1928

A.7.3.57.1	Test Purpose and Environment	1928

A.7.3.57.2	Test Requirements	1931

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1931

A.7.3.58.1	Test Purpose and Environment	1931

A.7.3.58.2	Test Requirements	1934

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1934

A.7.3.59.1	Test Purpose and Environment	1934

A.7.3.59.2	Test Requirements	1937

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1937

A.7.3.60.1	Test Purpose and Environment	1937

A.7.3.60.2	Test Requirements	1941

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1941

A.7.3.61.1	Test Purpose and Environment	1941

A.7.3.61.2	Test Requirements	1945

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1945

A.7.3.62.1	Test Purpose and Environment	1945

A.7.3.62.2	Test Requirements	1949

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1949

A.7.3.63.1	Test Purpose and Environment	1949

A.7.3.63.2	Test Requirements	1953

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1953

A.7.3.64.1	Test Purpose and Environment	1953

A.7.3.64.2	Test Requirements	1956

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1957

A.7.3.65.1	Test Purpose and Environment	1957

A.7.3.65.2	Test Requirements	1960

A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1961

A.7.3.66.1	Test Purpose and Environment	1961

A.7.3.66.2	Test Requirements	1963

A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1964

A.7.3.67.1	Test Purpose and Environment	1964

A.7.3.67.2	Test Requirements	1967

A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1968

A.7.3.68.1	Test Purpose and Environment	1968

A.7.3.68.2	Test Requirements	1970

A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1970

A.7.3.69.1	Test Purpose and Environment	1970

A.7.3.69.2	Test Requirements	1972

A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1972

A.7.3.70.1	Test Purpose and Environment	1972

A.7.3.70.2	Test Requirements	1974

A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1974

A.7.3.71.1	Test Purpose and Environment	1974

A.7.3.71.2	Test Requirements	1977

A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1977

A.7.3.72.1	Test Purpose and Environment	1977

A.7.3.72.2	Test Requirements	1979

A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1979

A.7.3.73.1	Test Purpose and Environment	1979

A.7.3.73.2	Test Requirements	1981

A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1981

A.7.3.74.1	Test Purpose and Environment	1981

A.7.3.74.2	Test Requirements	1984

A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1984

A.7.3.75.1	Test Purpose and Environment	1984

A.7.3.75.2	Test Requirements	1986

A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1987

A.7.3.76.1	Test Purpose and Environment	1987

A.7.3.76.2	Test Requirements	1990

A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1990

A.7.3.77.1	Test Purpose and Environment	1990

A.7.3.77.2	Test Requirements	1993

A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1993

A.7.3.78.1	Test Purpose and Environment	1993

A.7.3.78.2	Test Requirements	1996

A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1996

A.7.3.79.1	Test Purpose and Environment	1996

A.7.3.79.2	Test Requirements	1999

A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1999

A.7.3.80.1	Test Purpose and Environment	1999

A.7.3.80.2	Test Requirements	2002

A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	2002

A.7.3.81.1	Test Purpose and Environment	2002

A.7.3.81.2	Test Requirements	2005

A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	2005

A.7.3.82.1	Test Purpose and Environment	2005

A.7.3.82.2	Test Requirements	2008

A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	2008

A.7.3.83.1	Test Purpose and Environment	2008

A.7.3.83.2	Test Requirements	2009

A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	2010

A.7.3.84.1	Test Purpose and Environment	2010

A.7.3.84.2	Test Requirements	2012

A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	2012

A.7.3.85.1	Test Purpose and Environment	2012

A.7.3.85.2	Test Requirements	2013

A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	2014

A.7.3.86.1	Test Purpose and Environment	2014

A.7.3.86.2	Test Requirements	2016

A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	2016

A.7.3.87.1	Test Purpose and Environment	2016

A.7.3.87.2	Test Requirements	2018

A.7.3.88	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	2019

A.7.3.88.1	Test Purpose and Environment	2019

A.7.3.88.2	Test Requirements	2023

A.7.3.89	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	2023

A.7.3.89.1	Test Purpose and Environment	2023

A.7.3.89.2	Test Requirements	2027

A.7.3.90	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	2027

A.7.3.90.1	Test Purpose and Environment	2027

A.7.3.90.2	Test Requirements	2031

A.7.3.91	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	2031

A.7.3.91.1	Test Purpose and Environment	2031

A.7.3.91.2	Test Requirements	2035

A.7.3.92	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	2035

A.7.3.92.1	Test Purpose and Environment	2035

A.7.3.92.2	Test Requirements	2038

A.7.3.93	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	2039

A.7.3.93.1	Test Purpose and Environment	2039

A.7.3.93.2	Test Requirements	2042

A.7.3.94	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	2043

A.7.3.94.1	Test Purpose and Environment	2043

A.7.3.94.2	Test Requirements	2045

A.7.3.95	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	2046

A.7.3.95.1	Test Purpose and Environment	2046

A.7.3.95.2	Test Requirements	2049

A.7.3.96	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	2050

A.7.3.96.1	Test Purpose and Environment	2050

A.7.3.97	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	2051

A.7.3.97.1	Test Purpose and Environment	2051

A.7.3.97.2	Test Requirements	2054

A.7.3.98	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	2054

A.7.3.98.1	Test Purpose and Environment	2054

A.7.3.98.2	Test Requirements	2057

A.7.3.99	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	2057

A.7.3.99.1	Test Purpose and Environment	2057

A.7.3.99.2	Test Requirements	2060

A.7.3.100	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	2060

A.7.3.100.1	Test Purpose and Environment	2060

A.7.3.100.2	Test Requirements	2062

A.7.3.101	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	2062

A.7.3.101.1	Test Purpose and Environment	2062

A.7.3.101.2	Test Requirements	2064

A.7.4	Interruption for Dual Connectivity	2064

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	2064

A.7.4.1.1	Test Purpose and Environment	2064

A.7.4.1.2	Test Requirements	2067

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	2067

A.7.4.2.1	Test Purpose and Environment	2067

A.7.4.2.2	Test Requirements	2069

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	2069

A.7.4.3.1	Test Purpose and Environment	2069

A.7.4.3.2	Test Requirements	2071

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	2071

A.7.4.4.1	Test Purpose and Environment	2071

A.7.4.4.2	Test Requirements	2074

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	2074

A.7.4.5.1	Test Purpose and Environment	2074

A.7.4.5.2	Test Requirements	2076

A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching	2076

A.7.4.6.1	Test Purpose and Environment	2076

A.7.4.6.2	Test Requirements	2078

A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching	2078

A.7.4.7.1	Test Purpose and Environment	2078

A.7.4.7.2	Test Requirements	2082

A.7.5	Proximity-based Services	2082

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	2082

A.7.5.1.1	Test Purpose and Environment	2082

A.7.5.1.2	Test Requirements	2083

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	2084

A.7.5.2.1	Test Purpose and Environment	2084

A.7.5.1.2	Test Requirements	2084

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	2085

A.7.5.3.1	Test Purpose and Environment	2085

A.7.5.3.2	Test Requirements	2086

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	2087

A.7.5.4.1	Test Purpose and Environment	2087

A.7.5.4.2	Test Requirements	2088

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	2089

A.7.5.5.1	Test Purpose and Environment	2089

A.7.5.5.2	Test Requirements	2091

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	2092

A.7.5.6.1	Test Purpose and Environment	2092

A.7.5.6.2	Test Requirements	2093

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	2094

A.7.5.7.1	Test Purpose and Environment	2094

A.7.5.7.2	Test Requirements	2096

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	2096

A.7.5.8.1	Test Purpose and Environment	2096

A.7.5.8.2	Test Requirements	2098

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	2099

A.7.5.9.1	Test Purpose and Environment	2099

A.7.5.9.2	Test Requirements	2101

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	2102

A.7.5.10.1	Test Purpose and Environment	2102

A.7.5.10.2	Test Requirements	2104

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	2105

A.7.5.11.1	Test Purpose and Environment	2105

A.7.5.11.2	Test Requirements	2107

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	2107

A.7.5.12.1	Test Purpose and Environment	2107

A.7.5.12.2	Test Requirements	2111

A.7.6	Interruption for carrier aggregation	2112

A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching	2112

A.7.6.1.1	Test Purpose and Environment	2112

A.7.6.1.2	Test Requirements	2115

A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching	2115

A.7.6.2.1	Test Purpose and Environment	2115

A.7.6.2.2	Test Requirements	2118

A.8	UE Measurements Procedures	2099

A.8.1	E-UTRAN FDD Intra-frequency Measurements	2099

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2099

A.8.1.1.1	Test Purpose and Environment	2099

A.8.1.1.2	Test Requirements	2100

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	2101

A.8.1.2.1	Test Purpose and Environment	2101

A.8.1.2.2	Test Requirements	2102

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	2103

A.8.1.3.1	Test Purpose and Environment	2103

A.8.1.3.2	Test Requirements	2105

A.8.1.4	Void	2105

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2105

A.8.1.5.1	Test Purpose and Environment	2105

A.8.1.5.2	Test Requirements	2107

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2107

A.8.1.6.1	Test Purpose and Environment	2107

A.8.1.6.2	Test Requirements	2109

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2109

A.8.1.7.1	Test Purpose and Environment	2109

A.8.1.7.2	Test Requirements	2111

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2112

A.8.1.8.1	Test Purpose and Environment	2112

A.8.1.8.2	Test Requirements	2114

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	2115

A.8.1.9.1	Test Purpose and Environment	2115

A.8.1.9.2	Test Requirements	2115

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	2115

A.8.1.10.1	Test Purpose and Environment	2115

A.8.1.10.2	Test Requirements	2116

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	2116

A.8.1.11.1	Test Purpose and Environment	2116

A.8.1.11.2	Test Requirements	2118

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	2119

A.8.1.12.1	Test Purpose and Environment	2119

A.8.1.12.2	Test Requirements	2120

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	2121

A.8.1.13.1	Test Purpose and Environment	2121

A.8.1.13.2	Test Requirements	2123

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	2123

A.8.1.14.1	Test Purpose and Environment	2123

A.8.1.14.2	Test Requirements	2125

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	2126

A.8.1.15.1	Test Purpose and Environment	2126

A.8.1.15.2	Test Requirements	2127

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	2128

A.8.1.16.1	Test Purpose and Environment	2128

A.8.1.16.2	Test Requirements	2130

A.8.1.17	Void	2130

A.8.1.18	Void	2130

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	2130

A.8.1.19.1	Test Purpose and Environment	2130

A.8.1.19.2	Test Requirements	2132

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	2133

A.8.1.20.1	Test Purpose and Environment	2133

A.8.1.20.2	Test Requirements	2135

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	2135

A.8.1.21.1	Test Purpose and Environment	2135

A.8.1.21.2	Test Requirements	2137

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	2138

A.8.1.22.1	Test Purpose and Environment	2138

A.8.1.22.2	Test Requirements	2140

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	2140

A.8.1.23.1	Test Purpose and Environment	2140

A.8.1.23.2	Test Requirements	2141

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	2142

A.8.1.24.1	Test Purpose and Environment	2142

A.8.1.24.2	Test Requirements	2144

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	2144

A.8.1.25.1	Test Purpose and Environment	2144

A.8.1.25.2	Test Requirements	2146

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	2146

A.8.1.26.1	Test Purpose and Environment	2146

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	2149

A.8.1.27.1	Test Purpose and Environment	2149

A.8.1.27.2	Test Requirements	2151

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	2151

A.8.1.28.1	Test Purpose and Environment	2151

A.8.1.28.2	Test Requirements	2153

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	2153

A.8.1.29.1	Test Purpose and Environment	2153

A.8.1.29.2	Test Requirements	2155

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	2156

A.8.1.30.1	Test Purpose and Environment	2156

A.8.1.30.2	Test Requirements	2158

A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	2158

A.8.1.31.1	Test Purpose and Environment	2158

A.8.1.31.2	Test Requirements	2160

A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	2161

A.8.1.32.1	Test Purpose and Environment	2161

A.8.1.32.2	Test Requirements	2162

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	2163

A.8.1.33.1	Test Purpose and Environment	2163

A.8.1.33.2	Test Requirements	2164

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	2165

A.8.1.34.1	Test Purpose and Environment	2165

A.8.1.34.2	Test Requirements	2166

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	2167

A.8.1.35.1	Test Purpose and Environment	2167

A.8.1.35.2	Test Requirements	2168

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2169

A.8.1.36.1	Test Purpose and Environment	2169

A.8.1.36.2	Test Requirements	2170

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2171

A.8.1.37.1	Test Purpose and Environment	2171

A.8.1.37.2	Test Requirements	2173

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2173

A.8.1.38.1	Test Purpose and Environment	2173

A.8.1.38.2	Test Requirements	2174

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2175

A.8.1.39.1	Test Purpose and Environment	2175

A.8.1.39.2	Test Requirements	2177

A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	2177

A.8.1.40.1	Test Purpose and Environment	2177

A.8.1.40.2	Test Requirements	2179

A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2179

A.8.1.41.1	Test Purpose and Environment	2179

A.8.1.41.2	Test Requirement	2181

A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2181

A.8.1.42.1	Test Purpose and Environment	2181

A.8.1.42.2	Test Requirement	2182

A.8.1.43	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	2182

A.8.1.43.1	Test Purpose and Environment	2182

A.8.1.43.2	Test Requirements	2185

A.8.1.44	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2185

A.8.1.44.1	Test Purpose and Environment	2185

A.8.1.44.2	Test Requirements	2188

A.8.1.45	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2189

A.8.1.45.1	Test Purpose and Environment	2189

A.8.1.45.2	Test Requirements	2191

A.8.1.46	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2192

A.8.1.46.1	Test Purpose and Environment	2192

A.8.1.46.2	Test Requirements	2193

A.8.1.47	TDD Intra-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2194

A.8.1.47.1	Test Purpose and Environment	2194

A.8.1.47.2	Test Requirements	2196

A.8.1.48	TDD Intra-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2197

A.8.1.48.1	Test Purpose and Environment	2197

A.8.1.48.2	Test Requirements	2199

A.8.1.49	TDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2200

A.8.1.49.1	Test Purpose and Environment	2200

A.8.1.49.2	Test Requirements	2201

A.8.150	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2202

A.8.1.50.1	Test Purpose and Environment	2202

A.8.1.50.2	Test Requirements	2204

A.8.1.51	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2205

A.8.1.51.1	Test Purpose and Environment	2205

A.8.1.51.2	Test Requirements	2207

A.8.1.52	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2208

A.8.1.52.1	Test Purpose and Environment	2208

A.8.1.52.2	Test Requirements	2209

A.8.1.53	TDD Inter-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2210

A.8.1.53.1	Test Purpose and Environment	2210

A.8.1.53.2	Test Requirements	2212

A.8.1.54	TDD Inter-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2213

A.8.1.54.1	Test Purpose and Environment	2213

A.8.1.54.2	Test Requirements	2215

A.8.1.55	TDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2216

A.8.1.55.1	Test Purpose and Environment	2216

A.8.1.55.2	Test Requirements	2217

A.8.2	E-UTRAN TDD Intra-frequency Measurements	2218

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	2218

A.8.2.1.1	Test Purpose and Environment	2218

A.8.2.1.2	Test Requirements	2219

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	2219

A.8.2.2.1	Test Purpose and Environment	2219

A.8.2.2.2	Test Requirements	2222

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2222

A.8.2.3.1	Test Purpose and Environment	2222

A.8.2.3.2	Test Requirements	2224

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2225

A.8.2.4.1	Test Purpose and Environment	2225

A.8.2.4.2	Test Requirements	2227

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2227

A.8.2.5.1	Test Purpose and Environment	2227

A.8.2.5.2	Test Requirements	2229

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2230

A.8.2.6.1	Test Purpose and Environment	2230

A.8.2.6.2	Test Requirements	2233

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2234

A.8.2.7.1	Test Purpose and Environment	2234

A.8.2.7.2	Test Requirements	2235

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2236

A.8.2.8.1	Test Purpose and Environment	2236

A.8.2.8.2	Test Requirements	2238

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2238

A.8.2.9.1	Test Purpose and Environment	2238

A.8.2.9.2	Test Requirements	2240

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2241

A.8.2.10.1	Test Purpose and Environment	2241

A.8.2.10.2	Test Requirements	2243

A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	2243

A.8.2.11.1	Test Purpose and Environment	2243

A.8.2.11.2	Test Requirements	2245

A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	2245

A.8.2.12.1	Test Purpose and Environment	2245

A.8.2.12.2	Test Requirements	2247

A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	2248

A.8.2.13.1	Test Purpose and Environment	2248

A.8.2.13.2	Test Requirements	2250

A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2250

A.8.2.14.1	Test Purpose and Environment	2250

A.8.2.14.2	Test Requirement	2252

A.8.2.15	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	2252

A.8.2.15.1	Test Purpose and Environment	2252

A.8.2.15.2	Test Requirements	2254

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	2254

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2254

A.8.3.1.1	Test Purpose and Environment	2254

A.8.3.1.2	Test Requirements	2256

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2256

A.8.3.2.1	Test Purpose and Environment	2256

A.8.3.2.2	Test Requirements	2259

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	2259

A.8.3.3.1	Test Purpose and Environment	2259

A.8.3.3.2	Test Requirements	2262

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2262

A.8.3.4.1	Test Purpose and Environment	2262

A.8.3.4.2	Test Requirements	2264

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2264

A.8.3.5.2	Test Requirements	2266

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	2266

A.8.3.6.1	Test Purpose and Environment	2266

A.8.3.6.2	Test Requirements	2268

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2269

A.8.3.7.1	Test Purpose and Environment	2269

A.8.3.7.2	Test Requirements	2271

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2272

A.8.3.8.1	Test Purpose and Environment	2272

A.8.3.8.2	Test Requirements	2275

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2275

A.8.3.9.1	Test Purpose and Environment	2275

A.8.3.9.2	Test Requirements	2279

A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2280

A.8.3.10.1	Test Purpose and Environment	2280

A.8.3.10.2	Test Requirements	2281

A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap	2281

A.8.3.11.1	Test Purpose and Environment	2281

A.8.3.11.2	Test Requirement	2283

A.8.3.12	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2284

A.8.3.12.1	Test Purpose and Environment	2284

A.8.3.12.2	Test Requirement	2285

A.8.3.13	E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2286

A.8.3.13.1	Test Purpose and Environment	2286

A.8.3.13.2	Test Requirement	2287

A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2288

A.8.3.14.1	Test Purpose and Environment	2288

A.8.3.14.2	Test Requirement	2289

A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2290

A.8.3.15.1	Test Purpose and Environment	2290

A.8.3.15.2	Test Requirement	2291

A.8.3.16	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	2292

A.8.3.16.1	Test Purpose and Environment	2292

A.8.3.16.2	Test Requirement	2294

A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2294

A.8.3.17.1	Test Purpose and Environment	2294

A.8.3.17.2	Test Requirement	2297

A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2297

A.8.3.18.1	Test Purpose and Environment	2297

A.8.3.18.2	Test Requirement	2300

A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2300

A.8.3.19.1	Test Purpose and Environment	2300

A.8.3.19.2	Test Requirement	2303

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	2303

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2303

A.8.4.1.1	Test Purpose and Environment	2303

A.8.4.1.2	Test Requirements	2305

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	2305

A.8.4.2.1	Test Purpose and Environment	2305

A.8.4.2.2	Test Requirements	2308

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	2308

A.8.4.3.1	Test Purpose and Environment	2308

A.8.4.3.2	Test Requirements	2310

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2311

A.8.4.4.1	Test Purpose and Environment	2311

A.8.4.4.2	Test Requirements	2313

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2314

A.8.4.5.1	Test Purpose and Environment	2314

A.8.4.5.2	Test Requirements	2316

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	2316

A.8.4.6.1	Test Purpose and Environment	2316

A.8.4.6.2	Test Requirements	2317

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2317

A.8.4.7.1	Test Purpose and Environment	2317

A.8.4.7.2	Test Requirements	2321

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2321

A.8.4.8.1	Test Purpose and Environment	2321

A.8.4.8.2	Test Requirements	2324

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2324

A.8.4.9.1	Test Purpose and Environment	2324

A.8.4.9.2	Test Requirements	2329

A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2329

A.8.4.10.1	Test Purpose and Environment	2329

A.8.4.10.2	Test Requirements	2331

A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap	2331

A.8.4.11.1	Test Purpose and Environment	2331

A.8.4.11.2	Test Requirement	2333

A.8.4.12	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2333

A.8.4.12.1	Test Purpose and Environment	2333

A.8.4.12.2	Test Requirement	2335

A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2336

A.8.4.13.1	Test Purpose and Environment	2336

A.8.4.13.2	Test Requirement	2337

A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2338

A.8.4.14.1	Test Purpose and Environment	2338

A.8.4.14.2	Test Requirement	2340

A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2340

A.8.4.15.1	Test Purpose and Environment	2340

A.8.4.15.2	Test Requirement	2343

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	2343

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2343

A.8.5.1.1	Test Purpose and Environment	2343

A.8.5.1.2	Test Requirements	2345

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	2345

A.8.5.2.1	Test Purpose and Environment	2345

A.8.5.2.2	Test Requirements	2347

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	2347

A.8.5.3.1	Test Purpose and Environment	2347

A.8.5.3.2	Test Requirements	2350

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	2350

A.8.5.4.1	Test Purpose and Environment	2350

A.8.5.4.2	Test Requirements	2352

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2352

A.8.5.5.1	Test Purpose and Environment	2352

A.8.5.5.2	Test Requirements	2355

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	2355

A.8.5.6.1	Test Purpose and Environment	2355

A.8.5.6.2	Test Requirements	2356

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	2357

A.8.5.7.1	Test Purpose and Environment	2357

A.8.5.7.2	Test Requirements	2357

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2357

A.8.5.8.1	Test Purpose and Environment	2357

A.8.5.8.2	Test Requirements	2360

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	2360

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	2360

A.8.6.1.1	Test Purpose and Environment	2360

A.8.6.1.2	Test Requirements	2362

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2362

A.8.6.2.1	Test Purpose and Environment	2362

A.8.6.2.2	Test Requirements	2365

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2365

A.8.6.3.1	Test Purpose and Environment	2365

A.8.6.3.2	Test Requirements	2368

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	2368

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2368

A.8.7.1.1	Test Purpose and Environment	2368

A.8.7.1.1.1	Void	2368

A.8.7.1.1.2	1.28 Mcps TDD option	2368

A.8.7.1.1.3	Void	2370

A.8.7.1.2	Test Requirements	2370

A.8.7.1.2.1	Void	2370

A.8.7.1.2.2	1.28 Mcps TDD option	2370

A.8.7.1.2.3	Void	2370

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	2370

A.8.7.2.1	Test Purpose and Environment	2370

A.8.7.2.2	Test Requirements	2373

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	2374

A.8.7.3.1	Test Purpose and Environment	2374

A.8.7.3.2	Test Parameters	2374

A.8.7.3.3	Test Requirements	2375

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2376

A.8.7.4.1	Test Purpose and Environment	2376

A.8.7.4.2	Test Requirements	2378

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2378

A.8.7.5.1	Test Purpose and Environment	2378

A.8.7.5.2	Test Requirements	2380

A.8.7A	E-UTRAN FDD – UTRAN TDD Measurements	2380

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2380

A.8.7A.1.1	Test Purpose and Environment	2380

A.8.7A.1.2	Test Requirements	2383

A.8.8	E-UTRAN FDD – GSM Measurements	2383

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	2383

A.8.8.1.1	Test Purpose and Environment	2383

A.8.8.1.2	Test Requirements	2385

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	2385

A.8.8.2.1	Test Purpose and Environment	2385

A.8.8.2.2	Test Requirements	2387

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	2388

A.8.8.3.1	Test Purpose and Environment	2388

A.8.8.3.2	Test Requirements	2389

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	2390

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	2390

A.8.9.1.1	Test Purpose and Environment	2390

A.8.9.1.2	Test Requirements	2391

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2392

A.8.9.2.1	Test Purpose and Environment	2392

A.8.9.2.2	Test Requirements	2394

A.8.10	E-UTRAN TDD – GSM Measurements	2394

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	2394

A.8.10.1.1	Test Purpose and Environment	2394

A.8.10.1.2	Test Requirements	2395

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	2396

A.8.10.2.1	Test Purpose and Environment	2396

A.8.10.2.2	Test Requirements	2398

A.8.11	Monitoring of Multiple Layers	2398

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	2398

A.8.11.1.1	Test Purpose and Environment	2398

A.8.11.1.2	Test Requirements	2400

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	2401

A.8.11.2.1	Test Purpose and Environment	2401

A.8.11.2.2	Test Requirements	2402

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	2403

A.8.11.3.1	Test Purpose and Environment	2403

A.8.11.3.2	Test Requirements	2405

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	2405

A.8.11.4.1	Test Purpose and Environment	2405

A.8.11.4.2	Test Requirements	2408

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2408

A.8.11.5.1	Test Purpose and Environment	2408

A.8.11.5.2	Test Requirements	2410

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2411

A.8.11.6.1	Test Purpose and Environment	2411

A.8.11.6.2	Test Requirements	2413

A.8.12	RSTD Intra-frequency Measurements	2414

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	2414

A.8.12.1.1	Test Purpose and Environment	2414

A.8.12.1.2	Test Requirements	2418

A.8.12.1.2A	Test Requirements for UE Category 1bis	2418

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	2418

A.8.12.2.1	Test Purpose and Environment	2418

A.8.12.2.2	Test Requirements	2423

A.8.12.2.2A	Test Requirements for UE Category 1bis	2423

A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A	2423

A.8.12.3.1	Test Purpose and Environment	2423

A.8.12.3.2	Test Requirements	2429

A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A	2429

A.8.12.4.1	Test Purpose and Environment	2429

A.8.12.4.2	Test Requirements	2434

A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A	2434

A.8.12.5.1	Test Purpose and Environment	2434

A.8.12.5.2	Test Requirements	2439

A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B	2439

A.8.12.6.1	Test Purpose and Environment	2439

A.8.12.6.2	Test Requirements	2444

A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B	2444

A.8.12.7.1	Test Purpose and Environment	2444

A.8.12.7.2	Test Requirements	2449

A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B	2449

A.8.12.8.1	Test Purpose and Environment	2449

A.8.12.8.2	Test Requirements	2454

A.8.12.9	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2454

A.8.12.9.1	Test Purpose and Environment	2454

A.8.12.9.2	Test Requirements	2459

A.8.12.10	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2459

A.8.12.10.1	Test Purpose and Environment	2459

A.8.12.10.2	Test Requirements	2464

A.8.12.11	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2464

A.8.12.11.1	Test Purpose and Environment	2464

A.8.12.11.2	Test Requirements	2469

A.8.12.12	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2469

A.8.12.12.1	Test Purpose and Environment	2469

A.8.12.12.2	Test Requirements	2474

A.8.12.13	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2474

A.8.12.13.1	Test Purpose and Environment	2474

A.8.12.13.2	Test Requirements	2479

A.8.12.14	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2479

A.8.12.14.1	Test Purpose and Environment	2479

A.8.12.14.2	Test Requirements	2484

A.8.13	RSTD Inter-frequency Measurements	2484

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2484

A.8.13.1.1	Test Purpose and Environment	2484

A.8.13.1.2	Test Requirements	2489

A.8.13.1.2A	Test Requirements for UE Category 1bis	2489

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2489

A.8.13.2.1	Test Purpose and Environment	2489

A.8.13.2.2	Test Requirements	2495

A.8.13.2.2A	Test Requirements for UE Category 1bis	2495

A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A	2495

A.8.13.3.1	Test Purpose and Environment	2495

A.8.13.3.2	Test Requirements	2502

A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A	2502

A.8.13.4.1	Test Purpose and Environment	2502

A.8.13.4.2	Test Requirements	2508

A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A	2508

A.8.13.5.1	Test Purpose and Environment	2508

A.8.13.5.2	Test Requirements	2514

A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B	2514

A.8.13.6.1	Test Purpose and Environment	2514

A.8.13.6.2	Test Requirements	2520

A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B	2520

A.8.13.7.1	Test Purpose and Environment	2520

A.8.13.7.2	Test Requirements	2526

A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B	2526

A.8.13.8.1	Test Purpose and Environment	2526

A.8.13.8.2	Test Requirements	2532

A.8.13.9	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2532

A.8.13.9.1	Test Purpose and Environment	2532

A.8.13.9.2	Test Requirements	2538

A.8.13.10	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2538

A.8.13.10.1	Test Purpose and Environment	2538

A.8.13.10.2	Test Requirements	2544

A.8.13.11	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2544

A.8.13.11.1	Test Purpose and Environment	2544

A.8.13.11.2	Test Requirements	2550

A.8.13.12	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2550

A.8.13.12.1	Test Purpose and Environment	2550

A.8.13.12.2	Test Requirements	2556

A.8.13.13	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2556

A.8.13.13.1	Test Purpose and Environment	2556

A.8.13.13.2	Test Requirements	2562

A.8.13.14	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2562

A.8.13.14.1	Test Purpose and Environment	2562

A.8.13.14.2	Test Requirements	2568

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	2568

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2568

A.8.14.1.1	Test Purpose and Environment	2568

A.8.14.1.2	Test Requirements	2570

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2570

A.8.14.2.1	Test Purpose and Environment	2570

A.8.14.2.2	Test Requirements	2573

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2573

A.8.14.3.1	Test Purpose and Environment	2573

A.8.14.3.2	Test Requirements	2575

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	2576

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2576

A.8.15.1.1	Test Purpose and Environment	2576

A.8.15.1.2	Test Requirements	2577

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2577

A.8.15.2.1	Test Purpose and Environment	2577

A.8.15.2.2	Test Requirements	2580

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2580

A.8.15.3.1	Test Purpose and Environment	2580

A.8.15.3.2	Test Requirements	2582

A.8.16	E-UTRAN Carrier Aggregation Measurements	2583

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	2583

A.8.16.1.1	Test Purpose and Environment	2583

A.8.16.1.2	Test Requirements	2585

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	2585

A.8.16.2.1	Test Purpose and Environment	2585

A.8.16.2.2	Test Requirements	2587

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2587

A.8.16.3.1	Test Purpose and Environment	2587

A.8.16.3.2	Test Requirements	2589

A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX	2590

A.8.16.3A.1	Test Purpose and Environment	2590

A.8.16.3A.2	Test Requirements	2591

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	2592

A.8.16.4.1	Test Purpose and Environment	2592

A.8.16.4.2	Test Requirements	2593

A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2594

A.8.16.4A.1	Test Purpose and Environment	2594

A.8.16.4A.2	Test Requirements	2596

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2597

A.8.16.5.1	Test Purpose and Environment	2597

A.8.16.5.2	Test Requirements	2597

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2597

A.8.16.6.1	Test Purpose and Environment	2597

A.8.16.6.2	Test Requirements	2598

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2598

A.8.16.7.1	 Test Purpose and Environment	2598

A.8.16.7.2	 Test Requirements	2599

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2599

A.8.16.8.1	 Test Purpose and Environment	2599

A.8.16.8.2	 Test Requirements	2600

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2600

A.8.16.9.1	Test Purpose and Environment	2600

A.8.16.9.2	Test Requirements	2601

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2601

A.8.16.10.1	Test Purpose and Environment	2601

A.8.16.10.2	Test Requirements	2602

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2602

A.8.16.11.1	 Test Purpose and Environment	2602

A.8.16.11.2	 Test Requirements	2603

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2603

A.8.16.12.1	 Test Purpose and Environment	2603

A.8.16.12.2	 Test Requirements	2603

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	2603

A.8.16.13.1	Test Purpose and Environment	2603

A.8.16.13.2	Test Requirements	2604

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	2604

A.8.16.14.1	Test Purpose and Environment	2604

A.8.16.14.2	Test Requirements	2605

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	2605

A.8.16.15.1	 Test Purpose and Environment	2605

A.8.16.7.2	 Test Requirements	2605

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	2605

A.8.16.16.1	 Test Purpose and Environment	2605

A.8.16.16.2	 Test Requirements	2606

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	2606

A.8.16.17.1	Test Purpose and Environment	2606

A.8.16.17.2	Test Requirements	2608

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	2609

A.8.16.17A.1	Test Purpose and Environment	2609

A.8.16.17A.2	Test Requirements	2609

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2609

A.8.16.17B.1	Test Purpose and Environment	2609

A.8.16.17B.2	Test Requirements	2610

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2610

A.8.16.17C.1	Test Purpose and Environment	2610

A.8.16.17C.2	Test Requirements	2610

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	2610

A.8.16.18.1	Test Purpose and Environment	2610

A.8.16.18.2	Test Requirements	2612

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	2613

A.8.16.18A.1	Test Purpose and Environment	2613

A.8.16.18A.2	Test Requirements	2613

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2613

A.8.16.18B.1	Test Purpose and Environment	2613

A.8.16.18B.2	Test Requirements	2614

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2614

A.8.16.18C.1	Test Purpose and Environment	2614

A.8.16.18C.2	Test Requirements	2614

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	2614

A.8.16.18D.1	Test Purpose and Environment	2614

A.8.16.18D.2	Test Requirements	2615

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	2615

A.8.16.19.1	Test Purpose and Environment	2615

A.8.16.19.2	Test Requirements	2617

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2617

A.8.16.19A.1	Test Purpose and Environment	2617

A.8.16.19A.2	Test Requirements	2618

A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2618

A.8.16.19B.1	Test Purpose and Environment	2618

A.8.16.19B.2	Test Requirements	2618

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2618

A.8.16.19C.1	Test Purpose and Environment	2618

A.8.16.19C.2	Test Requirements	2619

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	2619

A.8.16.20.1	Test Purpose and Environment	2619

A.8.16.20.2	Test Requirements	2621

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2622

A.8.16.20A.1	Test Purpose and Environment	2622

A.8.16.20A.2	Test Requirements	2622

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2622

A.8.16.20B.1	Test Purpose and Environment	2622

A.8.16.20B.2	Test Requirements	2623

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2623

A.8.16.20C.1	Test Purpose and Environment	2623

A.8.16.20C.2	Test Requirements	2623

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	2623

A.8.16.20D.1	Test Purpose and Environment	2623

A.8.16.20D.2	Test Requirements	2624

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	2624

A.8.16.21.1	Test Purpose and Environment	2624

A.8.16.21.2	Test Requirements	2626

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	2626

A.8.16.22.1	Test Purpose and Environment	2626

A.8.16.22.2	Test Requirements	2628

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	2628

A.8.16.23.1	Test Purpose and Environment	2628

A.8.16.23.2	Test Requirements	2631

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	2631

A.8.16.24.1	Test Purpose and Environment	2631

A.8.16.24.2	Test Requirements	2634

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	2634

A.8.16.25.1	Test Purpose and Environment	2634

A.8.16.25.2	Test Requirements	2637

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	2637

A.8.16.26.1	Test Purpose and Environment	2637

A.8.16.26.2	Test Requirements	2640

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2640

A.8.16.27.1	Test Purpose and Environment	2640

A.8.16.27.2	Test Requirements	2645

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2645

A.8.16.28.1	Test Purpose and Environment	2645

A.8.16.28.2	Test Requirements	2650

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2650

A.8.16.29.1	Test Purpose and Environment	2650

A.8.16.29.2	Test Requirements	2655

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2655

A.8.16.30.1	Test Purpose and Environment	2655

A.8.16.30.2	Test Requirements	2660

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	2660

A.8.16.31.1	Test Purpose and Environment	2660

A.8.16.31.2	Test Requirements	2665

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	2665

A.8.16.32.1	Test Purpose and Environment	2665

A.8.16.32.2	Test Requirements	2670

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2670

A.8.16.33.1	Test Purpose and Environment	2670

A.8.16.33.2	Test Requirements	2675

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2675

A.8.16.34.1	Test Purpose and Environment	2675

A.8.16.34.2	Test Requirements	2680

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	2680

A.8.16.35.1	Test Purpose and Environment	2680

A.8.16.35.2	Test Requirements	2682

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	2683

A.8.16.36.1	Test Purpose and Environment	2683

A.8.16.36.2	Test Requirements	2685

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	2686

A.8.16.37.1	Test Purpose and Environment	2686

A.8.16.37.2	Test Requirements	2688

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	2689

A.8.16.38.1	Test Purpose and Environment	2689

A.8.16.38.2	Test Requirements	2691

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	2692

A.8.16.39.1	Test Purpose and Environment	2692

A.8.16.39.2	Test Requirements	2695

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	2695

A.8.16.40.1	Test Purpose and Environment	2695

A.8.16.40.2	Test Requirements	2698

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2698

A.8.16.41.1	Test Purpose and Environment	2698

A.8.16.41.2	Test Requirements	2700

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2701

A.8.16.42.1	Test Purpose and Environment	2701

A.8.16.42.2	Test Requirements	2703

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	2704

A.8.16.43.1	Test Purpose and Environment	2704

A.8.16.43.2	Test Requirements	2707

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	2707

A.8.16.44.1	Test Purpose and Environment	2707

A.8.16.44.2	Test Requirements	2710

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	2710

A.8.16.45.1	Test Purpose and Environment	2710

A.8.16.45.2	Test Requirements	2713

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	2713

A.8.16.46.1	Test Purpose and Environment	2713

A.8.16.46.2	Test Requirements	2715

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2716

A.8.16.47.1	Test Purpose and Environment	2716

A.8.16.47.2	Test Requirements	2718

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2719

A.8.16.48.1	Test Purpose and Environment	2719

A.8.16.48.2	Test Requirements	2721

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2722

A.8.16.49.1	Test Purpose and Environment	2722

A.8.16.49.2	Test Requirements	2725

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2725

A.8.16.50.1	Test Purpose and Environment	2725

A.8.16.50.2	Test Requirements	2727

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2728

A.8.16.51.1	Test Purpose and Environment	2728

A.8.16.51.2	Test Requirements	2733

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2733

A.8.16.52.1	Test Purpose and Environment	2733

A.8.16.52.2	Test Requirements	2738

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2738

A.8.16.53.1	Test Purpose and Environment	2738

A.8.16.53.2	Test Requirements	2742

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2742

A.8.16.54.1	Test Purpose and Environment	2742

A.8.16.54.2	Test Requirements	2746

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2746

A.8.16.55.1	Test Purpose and Environment	2746

A.8.16.55.2	Test Requirements	2752

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2752

A.8.16.56.1	Test Purpose and Environment	2752

A.8.16.56.2	Test Requirements	2758

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	2758

A.8.16.57.1	Test Purpose and Environment	2758

A.8.16.57.2	Test Requirements	2760

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	2761

A.8.16.58.1	Test Purpose and Environment	2761

A.8.16.58.2	Test Requirements	2763

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2764

A.8.16.59.1	Test Purpose and Environment	2764

A.8.16.59.2	Test Requirements	2768

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2769

A.8.16.60.1	Test Purpose and Environment	2769

A.8.16.60.2	Test Requirements	2772

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2773

A.8.16.61.1	Test Purpose and Environment	2773

A.8.16.61.2	Test Requirements	2776

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2776

A.8.16.62.1	Test Purpose and Environment	2776

A.8.16.62.2	Test Requirements	2778

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2779

A.8.16.63.1	Test Purpose and Environment	2779

A.8.16.63.2	Test Requirements	2782

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2783

A.8.16.64.1	Test Purpose and Environment	2783

A.8.16.64.2	Test Requirements	2787

A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2787

A.8.16.65.1	Test Purpose and Environment	2787

A.8.16.65.2	Test Requirements	2792

A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2792

A.8.16.66.1	Test Purpose and Environment	2792

A.8.16.66.2	Test Requirements	2797

A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	2797

A.8.16.67.1	Test Purpose and Environment	2797

A.8.16.67.2	Test Requirements	2802

A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	2802

A.8.16.68.1	Test Purpose and Environment	2802

A.8.16.68.2	Test Requirements	2807

A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2807

A.8.16.69.1	Test Purpose and Environment	2807

A.8.16.69.2	Test Requirements	2811

A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2811

A.8.16.70.1	Test Purpose and Environment	2811

A.8.16.70.2	Test Requirements	2815

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2815

A.8.16.71.1	Test Purpose and Environment	2815

A.8.16.71.2	Test Requirements	2820

A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2820

A.8.16.72.1	Test Purpose and Environment	2820

A.8.16.72.2	Test Requirements	2825

A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2825

A.8.16.73.1	Test Purpose and Environment	2825

A.8.16.73.2	Test Requirements	2832

A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2832

A.8.16.74.1	Test Purpose and Environment	2832

A.8.16.74.2	Test Requirements	2838

A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX	2838

A.8.16.75.1	Test Purpose and Environment	2838

A.8.16.75.2	Test Requirements	2844

A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX	2844

A.8.16.76.1	Test Purpose and Environment	2844

A.8.16.76.2	Test Requirements	2850

A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX	2850

A.8.16.77.1	Test Purpose and Environment	2850

A.8.16.77.2	Test Requirements	2854

A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX	2854

A.8.16.78.1	Test Purpose and Environment	2854

A.8.16.78.2	Test Requirements	2856

A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2857

A.8.16.79.1	Test Purpose and Environment	2857

A.8.16.79.2	Test Requirements	2864

A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2864

A.8.16.80.1	Test Purpose and Environment	2864

A.8.16.80.2	Test Requirements	2871

A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2871

A.8.16.81.1	Test Purpose and Environment	2871

A.8.16.81.2	Test Requirements	2877

A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2877

A.8.16.82.1	Test Purpose and Environment	2877

A.8.16.82.2	Test Requirements	2883

A.8.16.83	3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2883

A.8.16.83.1	Test Purpose and Environment	2883

A.8.16.83.2	Test Requirements	2888

A.8.16.84	3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2888

A.8.16.84.1	Test Purpose and Environment	2888

A.8.16.84.2	Test Requirements	2894

A.8.16.85	3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2894

A.8.16.85.1	Test Purpose and Environment	2894

A.8.16.85.2	Test Requirements	2898

A.8.16.86	3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2898

A.8.16.86.1	Test Purpose and Environment	2898

A.8.16.86.2	Test Requirements	2901

A.8.16.87	4 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2901

A.8.16.87.1	Test Purpose and Environment	2901

A.8.16.87.2	Test Requirements	2907

A.8.16.88	4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2907

A.8.16.88.1	Test Purpose and Environment	2907

A.8.16.88.2	Test Requirements	2915

A.8.16.89	4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2915

A.8.16.89.1	Test Purpose and Environment	2915

A.8.16.89.2	Test Requirements	2922

A.8.16.90	4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2922

A.8.16.90.1	Test Purpose and Environment	2922

A.8.16.90.2	Test Requirements	2929

A.8.16.91	5 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2929

A.8.16.91.1	Test Purpose and Environment	2929

A.8.16.91.2	Test Requirements	2935

A.8.16.92	5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2935

A.8.16.92.1	Test Purpose and Environment	2935

A.8.16.92.2	Test Requirements	2943

A.8.16.93	5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2943

A.8.16.93.1	Test Purpose and Environment	2943

A.8.16.93.2	Test Requirements	2950

A.8.16.94	5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2950

A.8.16.94.1	Test Purpose and Environment	2950

A.8.16.94.2	Test Requirements	2957

A.8.16.95	6 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2957

A.8.16.95.1	Test Purpose and Environment	2957

A.8.16.95.2	Test Requirements	2965

A.8.16.96	6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2965

A.8.16.96.1	Test Purpose and Environment	2965

A.8.16.96.2	Test Requirements	2974

A.8.16.97	6 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2974

A.8.16.97.1	Test Purpose and Environment	2974

A.8.16.97.2	Test Requirements	2981

A.8.16.98	6 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2981

A.8.16.98.1	Test Purpose and Environment	2981

A.8.16.98.2	Test Requirements	2989

A.8.16.99	7 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2989

A.8.16.99.1	Test Purpose and Environment	2989

A.8.16.99.2	Test Requirements	2999

A.8.16.100	7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2999

A.8.16.100.1	Test Purpose and Environment	2999

A.8.16.100.2	Test Requirements	3009

A.8.16.101	7 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	3009

A.8.16.101.1	Test Purpose and Environment	3009

A.8.16.101.2	Test Requirements	3019

A.8.16.102	7 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	3019

A.8.16.102.1	Test Purpose and Environment	3019

A.8.16.102.2	Test Requirements	3030

A.8.16.103	Hibernation and Activation of Known SCell in Non-DRX with generic duplex modes	3030

A.8.16.103.1	Test Purpose and Environment	3030

A.8.16.103.2	Test Requirements	3035

A.8.16.104	Hibernation and Activation of Unknown SCell in Non-DRX with generic duplex modes	3036

A.8.16.104.1	Test Purpose and Environment	3036

A.8.16.104.2	Test Requirements	3040

A.8.16.105	Idle Mode measurements of inter-frequency CA candidate cells for early reporting	3041

A.8.16.105.1	Test Purpose and Environment	3041

A.8.16.105.2	Test Requirements	3045

A.8.16.106	Direct Activation of Known SCell in Non-DRX with generic duplex modes	3046

A.8.16.106.1	Test Purpose and Environment	3046

A.8.16.106.2	Test Requirements	3050

A.8.16.107	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	3050

A.8.16.107.1	Test Purpose and Environment	3050

A.8.16.107.2	Test Requirements	3052

A.8.16.108	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	3053

A.8.16.108.1	Test Purpose and Environment	3053

A.8.16.108.2	Test Requirements	3055

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	3056

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	3056

A.8.17.1.1	Test Purpose and Environment	3056

A.8.17.1.2	Test Requirements	3062

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	3062

A.8.17.2.1	Test Purpose and Environment	3062

A.8.17.2.2	Test Requirements	3069

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	3069

A.8.17.3.1	Test Purpose and Environment	3069

A.8.17.3.2	Test Requirements	3070

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	3070

A.8.17.4.1	Test Purpose and Environment	3070

A.8.17.4.2	Test Requirements	3071

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	3071

A.8.17.5.1	Test Purpose and Environment	3071

A.8.17.5.2	Test Requirements	3072

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	3072

A.8.17.6.1	Test Purpose and Environment	3072

A.8.17.6.2	Test Requirements	3073

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	3074

A.8.17.7.1	Test Purpose and Environment	3074

A.8.17.7.2	Test Requirements	3074

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	3075

A.8.17.8.1	Test Purpose and Environment	3075

A.8.17.8.2	Test Requirements	3075

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	3076

A.8.17.9.1	Test Purpose and Environment	3076

A.8.17.9.2	Test Requirements	3077

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	3077

A.8.17.10.1	Test Purpose and Environment	3077

A.8.17.10.2	Test Requirements	3084

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	3085

A.8.17.11.1	Test Purpose and Environment	3085

A.8.17.11.2	Test Requirements	3091

A.8.18	E-UTRAN TDD – HRPD Measurements	3092

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	3092

A.8.18.1.1	Test Purpose and Environment	3092

A.8.18.1.2	Test Requirements	3094

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	3094

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	3094

A.8.19.1.1	Test Purpose and Environment	3094

A.8.19.1.2	Test Requirements	3095

A.8.20	Inter-frequency/RAT Measurements in CA mode	3096

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	3096

A.8.20.1.1	Test Purpose and Environment	3096

A.8.20.1.2	Test Requirements	3097

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3097

A.8.20.2.1	Test Purpose and Environment	3098

A.8.20.2.2	Test Requirements	3099

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	3100

A.8.20.2A.1	Test Purpose and Environment	3100

A.8.20.2A.2	Test Requirements	3100

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	3100

A.8.20.2B.1	Test Purpose and Environment	3100

A.8.20.2B.2	Test Requirements	3103

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	3103

A.8.20.3.1	Test Purpose and Environment	3103

A.8.20.3.2	Test Requirements	3105

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	3105

A.8.20.4.1	Test Purpose and Environment	3105

A.8.20.4.1.1	1.28 Mcps TDD option	3105

A.8.20.4.2	Test Requirements	3107

A.8.20.4.2.1	1.28 Mcps TDD option	3107

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	3107

A.8.20.4A.1	Test Purpose and Environment	3107

A.8.20.4A.1.1	1.28 Mcps TDD option	3107

A.8.20.4A.2	Test Requirements	3107

A.8.20.4A.2.1	1.28 Mcps TDD option	3107

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	3108

A.8.20.4B.1	Test Purpose and Environment	3108

A.8.20.4B.1.1	1.28 Mcps TDD option	3108

A.8.20.4B.2	Test Requirements	3110

A.8.20.4B.2.1	1.28 Mcps TDD option	3110

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	3110

A.8.21.1	Test Purpose and Environment	3110

A.8.21.2 Test Requirements	3114

A.8.22	E-UTRAN Discovery Signal Measurements	3114

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	3114

A.8.22.1.1	Test Purpose and Environment	3114

A.8.22.1.2	Test Requirements	3117

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	3117

A.8.22.2.1	Test Purpose and Environment	3117

A.8.22.2.2	Test Requirements	3120

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	3120

A.8.22.3.1	Test Purpose and Environment	3120

A.8.22.3.2	Test Requirements	3123

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	3123

A.8.22.4.1	Test Purpose and Environment	3123

A.8.22.4.2	Test Requirements	3126

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	3126

A.8.22.5.1	Test Purpose and Environment	3126

A.8.22.5.2	Test Requirements	3130

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	3130

A.8.22.6.1	Test Purpose and Environment	3130

A.8.22.6.2	Test Requirements	3134

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	3134

A.8.22.7.1	Test Purpose and Environment	3134

A.8.22.7.2	Test Requirements	3138

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	3138

A.8.22.8.1	Test Purpose and Environment	3138

A.8.22.8.2	Test Requirements	3142

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	3142

A.8.22.9.1	Test Purpose and Environment	3142

A.8.22.9.2	Test Requirements	3144

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	3145

A.8.22.10.1	Test Purpose and Environment	3145

A.8.22.10.2	Test Requirements	3147

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	3148

A.8.22.11.1	Test Purpose and Environment	3148

A.8.22.11.2	Test Requirements	3151

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	3151

A.8.22.12.1	Test Purpose and Environment	3151

A.8.22.12.2	Test Requirements	3155

A.8.23	E-UTRAN Dual Connectivity Measurements	3155

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	3155

A.8.23.1.1	Test Purpose and Environment	3155

A.8.23.1.2	Test Requirements	3158

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	3158

A.8.23.2.1	Test Purpose and Environment	3158

A.8.23.2.2	Test Requirements	3161

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	3161

A.8.23.3.1	Test Purpose and Environment	3161

A.8.23.3.2	Test Requirements	3164

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	3164

A.8.23.4.1	Test Purpose and Environment	3164

A.8.23.4.2	Test Requirements	3167

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	3167

A.8.23.5.1	Test Purpose and Environment	3167

A.8.23.5.2	Test Requirements	3170

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	3170

A.8.23.6.1	Test Purpose and Environment	3170

A.8.23.6.2	Test Requirements	3173

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	3173

A.8.23.7.1	Test Purpose and Environment	3173

A.8.23.7.2	Test Requirements	3175

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	3176

A.8.23.8.1	Test Purpose and Environment	3176

A.8.23.8.2	Test Requirements	3178

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	3179

A.8.23.9.1	Test Purpose and Environment	3179

A.8.23.9.2	Test Requirements	3182

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	3182

A.8.23.10.1	Test Purpose and Environment	3182

A.8.23.10.2	Test Requirements	3185

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	3185

A.8.23.11.1	Test Purpose and Environment	3185

A.8.23.11.2	Test Requirements	3188

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	3188

A.8.23.12.1	Test Purpose and Environment	3188

A.8.23.12.2	Test Requirements	3191

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	3191

A.8.23.13.1	Test Purpose and Environment	3191

A.8.23.13.2	Test Requirements	3194

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	3194

A.8.23.14.1	Test Purpose and Environment	3194

A.8.23.14.2	Test Requirements	3197

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	3197

A.8.23.15.1	Test Purpose and Environment	3197

A.8.23.15.2	Test Requirements	3200

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	3200

A.8.23.16.1	Test Purpose and Environment	3200

A.8.23.16.2	Test Requirements	3201

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	3202

A.8.23.17.1	Test Purpose and Environment	3202

A.8.23.17.2	Test Requirements	3204

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3204

A.8.23.18.1	Test Purpose and Environment	3204

A.8.23.18.2	Test Requirements	3205

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	3206

A.8.23.19.1	Test Purpose and Environment	3206

A.8.23.19.2	Test Requirements	3207

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3208

A.8.23.20.1	Test Purpose and Environment	3208

A.8.23.20.2	Test Requirements	3209

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3210

A.8.23.21.1	Test Purpose and Environment	3210

A.8.23.21.2	Test Requirements	3211

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	3212

A.8.23.22.1	Test Purpose and Environment	3212

A.8.23.22.2	Test Requirements	3213

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3214

A.8.23.23.1	Test Purpose and Environment	3214

A.8.23.23.2	Test Requirements	3215

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	3216

A.8.23.24.1	Test Purpose and Environment	3216

A.8.23.24.2	Test Requirements	3218

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	3219

A.8.23.25.1	Test Purpose and Environment	3219

A.8.23.25.2	Test Requirements	3221

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	3222

A.8.23.26.1	Test Purpose and Environment	3222

A.8.23.26.2	Test Requirements	3224

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	3225

A.8.23.27.1	Test Purpose and Environment	3225

A.8.23.27.2	Test Requirements	3228

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	3228

A.8.23.28.1	Test Purpose and Environment	3228

A.8.23.28.2	Test Requirements	3232

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	3232

A.8.23.29.1	Test Purpose and Environment	3232

A.8.23.29.2	Test Requirements	3236

A.8.24	Proximity-based Services	3236

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	3236

A.8.24.1.1	Test Purpose and Environment	3236

A.8.24.1.2	Test Requirements	3237

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	3238

A.8.24.2.1	Test Purpose and Environment	3238

A.8.24.2.2	Test Requirements	3239

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	3239

A.8.24.3.1	Test Purpose and Environment	3240

A.8.24.3.2	Test Requirements	3241

A.8.25	E-UTRAN-WLAN Measurements	3242

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	3242

A.8.25.1.1	Test Purpose and Environment	3242

A.8.25.1.2	Test Requirements	3244

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	3244

A.8.25.2.1	Test Purpose and Environment	3244

A.8.25.2.2	Test Requirements	3246

A.8.26 Frame Structure 3 (FS3)	3246

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	3246

A.8.26.1.1	Test Purpose and Environment	3246

A.8.26.1.2	Test Requirements	3248

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	3249

A.8.26.2.1	Test Purpose and Environment	3249

A.8.26.2.2	Test Requirements	3251

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3252

A.8.26.3.1	Test Purpose and Environment	3252

A.8.26.3.2	Test Requirements	3255

A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3255

A.8.26.3A.1	Test Purpose and Environment	3255

A.8.26.3A.2	Test Requirements	3258

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	3258

A.8.26.4.1	Test Purpose and Environment	3258

A.8.26.4.2	Test Requirements	3262

A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3262

A.8.26.4A.1	Test Purpose and Environment	3262

A.8.26.4A.2	Test Requirements	3266

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	3266

A.8.26.5.1	Test Purpose and Environment	3266

A.8.26.5.2	Test Requirements	3269

A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	3269

A.8.26.5A.1	Test Purpose and Environment	3269

A.8.26.5A.2	Test Requirements	3273

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	3273

A.8.26.6.1	Test Purpose and Environment	3273

A.8.26.6.2	Test Requirements	3277

A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	3277

A.8.26.6A.1	Test Purpose and Environment	3277

A.8.26.6A.2	Test Requirements	3281

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	3281

A.8.26.7.1	Test Purpose and Environment	3281

A.8.26.7.2	Test Requirements	3284

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	3284

A.8.26.8.1	Test Purpose and Environment	3284

A.8.26.8.2	Test Requirements	3288

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3288

A.8.26.9.1	Test Purpose and Environment	3288

A.8.26.9.2	Test Requirements	3291

A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3291

A.8.26.10.1	Test Purpose and Environment	3291

A.8.26.10.2	Test Requirements	3294

A.9	Measurement Performance Requirements	3295

A.9.1	RSRP	3295

A.9.1.1	FDD Intra frequency case	3295

A.9.1.1.1	Test Purpose and Environment	3295

A.9.1.1.2	Test parameters	3295

A.9.1.1.3	Test Requirements	3298

A.9.1.2	TDD Intra frequency case	3298

A.9.1.2.1	Test Purpose and Environment	3298

A.9.1.2.2	Test parameters	3298

A.9.1.2.3	Test Requirements	3299

A.9.1.3	FDD—FDD Inter frequency case	3300

A.9.1.3.1	Test Purpose and Environment	3300

A.9.1.3.2	Test parameters	3300

A.9.1.3.3	Test Requirements	3303

A.9.1.4	TDD—TDD Inter frequency case	3303

A.9.1.4.1	Test Purpose and Environment	3303

A.9.1.4.2	Test parameters	3303

A.9.1.4.3	Test Requirements	3305

A.9.1.5	FDD—TDD Inter frequency case	3306

A.9.1.5.1	Test Purpose and Environment	3306

A.9.1.5.2	Test parameters	3306

A.9.1.5.3	Test Requirements	3307

A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation	3308

A.9.1.6.1	Test Purpose and Environment	3308

A.9.1.6.2	Test parameters	3308

A.9.1.6.3	Test Requirements	3311

A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation	3311

A.9.1.7.1	Test Purpose and Environment	3311

A.9.1.7.2	Test parameters	3311

A.9.1.7.3	Test Requirements	3314

A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3314

A.9.1.8.1	Test Purpose and Environment	3314

A.9.1.8.2	Test parameters	3314

A.9.1.8.3	Test Requirements	3318

A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3318

A.9.1.9.1	Test Purpose and Environment	3318

A.9.1.9.2	Test parameters	3318

A.9.1.9.3	Test Requirements	3321

A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	3321

A.9.1.10.1	Test Purpose and Environment	3321

A.9.1.10.2	Test parameters	3321

A.9.1.10.3	Test Requirements	3324

A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	3324

A.9.1.11.1	Test Purpose and Environment	3324

A.9.1.11.2	Test parameters	3324

A.9.1.11.3	Test Requirements	3328

A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	3328

A.9.1.12.1	Test Purpose and Environment	3328

A.9.1.12.2	Test parameters	3328

A.9.1.12.3	Test Requirements	3329

A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	3329

A.9.1.13.1	Test Purpose and Environment	3329

A.9.1.13.2	Test parameters	3329

A.9.1.13.3	Test Requirements	3330

A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3330

A.9.1.14.1	Test Purpose and Environment	3330

A.9.1.14.2	Test parameters	3330

A.9.1.14.3	Test Requirements	3334

A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3334

A.9.1.15.1	Test Purpose and Environment	3334

A.9.1.15.2	Test parameters	3334

A.9.1.15.3	Test Requirements	3336

A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth	3337

A.9.1.16.1	Test Purpose and Environment	3337

A.9.1.16.2	Test parameters	3337

A.9.1.16.3	Test Requirements	3338

A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth	3338

A.9.1.17.1	Test Purpose and Environment	3338

A.9.1.17.2	Test parameters	3339

A.9.1.17.3	Test Requirements	3340

A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	3340

A.9.1.18.1	Test Purpose and Environment	3340

A.9.1.18.2	Test parameters	3340

A.9.1.18.3	Test Requirements	3341

A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	3342

A.9.1.19.1	Test Purpose and Environment	3342

A.9.1.19.2	Test parameters	3342

A.9.1.19.3	Test Requirements	3342

A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	3342

A.9.1.20.1	Test Purpose and Environment	3342

A.9.1.20.2	Test parameters	3342

A.9.1.20.3	Test Requirements	3344

A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	3344

A.9.1.21.1	Test Purpose and Environment	3344

A.9.1.21.2	Test parameters	3344

A.9.1.21.3	Test Requirements	3344

A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	3344

A.9.1.22.1	Test Purpose and Environment	3344

A.9.1.22.2	Test parameters	3345

A.9.1.22.3	Test Requirements	3349

A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	3349

A.9.1.23.1	Test Purpose and Environment	3349

A.9.1.23.2	Test parameters	3349

A.9.1.23.3	Test Requirements	3353

A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz	3353

A.9.1.24.1	Test Purpose and Environment	3353

A.9.1.24.2	Test parameters	3353

A.9.1.24.3	Test Requirements	3354

A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3354

A.9.1.25.1	Test Purpose and Environment	3354

A.9.1.25.2	Test parameters	3354

A.9.1.25.3	Test Requirements	3357

A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3357

A.9.1.26.1	Test Purpose and Environment	3357

A.9.1.26.2	Test parameters	3357

A.9.1.26.3	Test Requirements	3360

A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3360

A.9.1.27.1	Test Purpose and Environment	3360

A.9.1.27.2	Test parameters	3360

A.9.1.27.3	Test Requirements	3363

A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal	3363

A.9.1.28.1	Test Purpose and Environment	3363

A.9.1.28.2	Test parameters	3363

A.9.1.28.3	Test Requirements	3366

A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3366

A.9.1.29.1	Test Purpose and Environment	3366

A.9.1.29.2	Test parameters	3366

A.9.1.29.3	Test Requirements	3369

A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3369

A.9.1.30.1	Test Purpose and Environment	3369

A.9.1.30.2	Test parameters	3370

A.9.1.30.3	Test Requirements	3373

A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3373

A.9.1.31.1	Test Purpose and Environment	3373

A.9.1.31.2	Test parameters	3373

A.9.1.31.3	Test Requirements	3376

A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal	3377

A.9.1.32.1	Test Purpose and Environment	3377

A.9.1.32.2	Test parameters	3377

A.9.1.32.3	Test Requirements	3380

A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	3380

A.9.1.33.1	Test Purpose and Environment	3380

A.9.1.33.2	Test parameters	3380

A.9.1.33.3	Test Requirements	3383

A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	3383

A.9.1.34.1	Test Purpose and Environment	3383

A.9.1.34.2	Test parameters	3383

A.9.1.34.3	Test Requirements	3386

A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	3386

A.9.1.35.1	Test Purpose and Environment	3386

A.9.1.35.2	Test parameters	3386

A.9.1.35.3	Test Requirements	3389

A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	3390

A.9.1.36.1	Test Purpose and Environment	3390

A.9.1.36.2	Test parameters	3390

A.9.1.36.3	Test Requirements	3393

A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	3393

A.9.1.37.1	Test Purpose and Environment	3393

A.9.1.37.2	Test parameters	3393

A.9.1.37.3	Test Requirements	3396

A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	3397

A.9.1.38.1	Test Purpose and Environment	3397

A.9.1.38.2	Test parameters	3397

A.9.1.38.3	Test Requirements	3400

A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3401

A.9.1.39.1	Test Purpose and Environment	3401

A.9.1.39.2	Test parameters	3401

A.9.1.39.3	Test Requirements	3407

A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3407

A.9.1.40.1	Test Purpose and Environment	3407

A.9.1.40.2	Test parameters	3408

A.9.1.40.3	Test Requirements	3414

A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0	3414

A.9.1.41.1	Test Purpose and Environment	3414

A.9.1.41.2	Test parameters	3414

A.9.1.41.3	Test Requirements	3417

A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0	3417

A.9.1.42.1	Test Purpose and Environment	3417

A.9.1.42.2	Test parameters	3417

A.9.1.42.3	Test Requirements	3420

A.9.1.43	TDD RSRP Intra frequency case for UE category 0	3420

A.9.1.43.1	Test Purpose and Environment	3420

A.9.1.43.2	Test parameters	3420

A.9.1.43.3	Test Requirements	3421

A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	3422

A.9.1.44.1	Test Purpose and Environment	3422

A.9.1.44.2	Test parameters	3422

A.9.1.44.3	Test Requirements	3427

A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	3427

A.9.1.45.1	Test Purpose and Environment	3427

A.9.1.45.2	Test parameters	3427

A.9.1.45.3	Test Requirements	3432

A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3432

A.9.1.46.1	Test Purpose and Environment	3432

A.9.1.46.2	Test parameters	3432

A.9.1.46.3	Test Requirements	3441

A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3442

A.9.1.47.1	Test Purpose and Environment	3442

A.9.1.47.2	Test parameters	3442

A.9.1.47.3	Test Requirements	3451

A.9.1.48	5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	3451

A.9.1.48.1	Test Purpose and Environment	3451

A.9.1.48.2	Test parameters	3452

A.9.1.48.3	Test Requirements	3462

A.9.1.49	5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	3463

A.9.1.49.1	Test Purpose and Environment	3463

A.9.1.49.2	Test parameters	3463

A.9.1.49.3	Test Requirements	3473

A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3474

A.9.1.50.1	Test Purpose and Environment	3474

A.9.1.50.2	Test parameters	3474

A.9.1.50.3	Test Requirements	3481

A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3482

A.9.1.51.1	Test Purpose and Environment	3482

A.9.1.51.2	Test parameters	3482

A.9.1.51.3	Test Requirements	3488

A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	3489

A.9.1.52.1	Test Purpose and Environment	3489

A.9.1.52.2	Test parameters	3489

A.9.1.52.3	Test Requirements	3492

A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	3492

A.9.1.52A.1	Test Purpose and Environment	3492

A.9.1.52A.2	Test parameters	3492

A.9.1.52A.3	Test Requirements	3493

A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	3493

A.9.1.53.1	Test Purpose and Environment	3493

A.9.1.53.2	Test parameters	3494

A.9.1.53.3	Test Requirements	3497

A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	3497

A.9.1.53A.1	Test Purpose and Environment	3497

A.9.1.53A.2	Test parameters	3497

A.9.1.53A.3	Test Requirements	3498

A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	3499

A.9.1.54.1	Test Purpose and Environment	3499

A.9.1.54.2	Test parameters	3499

A.9.1.54.3	Test Requirements	3501

A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell	3501

A.9.1.55.1	Test Purpose and Environment	3501

A.9.1.55.2	Test parameters	3501

A.9.1.55.3	Test Requirements	3505

A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell	3505

A.9.1.56.1	Test Purpose and Environment	3505

A.9.1.56.2	Test parameters	3505

A.9.1.56.3	Test Requirements	3508

A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	3509

A.9.1.57.1	Test Purpose and Environment	3509

A.9.1.57.2	Test parameters	3509

A.9.1.57.3	Test Requirements	3512

A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	3512

A.9.1.57A.1	Test Purpose and Environment	3512

A.9.1.57A.2	Test parameters	3512

A.9.1.57A.3	Test Requirements	3513

A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	3513

A.9.1.58.1	Test Purpose and Environment	3513

A.9.1.58.2	Test parameters	3514

A.9.1.58.3	Test Requirements	3517

A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	3517

A.9.1.58A.1	Test Purpose and Environment	3517

A.9.1.58A.2	Test parameters	3517

A.9.1.58A.3	Test Requirements	3518

A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	3518

A.9.1.59.1	Test Purpose and Environment	3518

A.9.1.59.2	Test parameters	3519

A.9.1.59.3	Test Requirements	3521

A.9.1.60	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell	3521

A.9.1.60.1	Test Purpose and Environment	3521

A.9.1.60.2	Test parameters	3521

A.9.1.60.3	Test Requirements	3524

A.9.1.61	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell	3525

A.9.1.61.1	Test Purpose and Environment	3525

A.9.1.61.2	Test parameters	3525

A.9.1.61.3	Test Requirements	3528

A.9.1.62	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	3528

A.9.1.62.1	Test Purpose and Environment	3528

A.9.1.62.2	Test parameters	3528

A.9.1.62.3	Test Requirements	3531

A.9.1.63	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	3531

A.9.1.53.1	Test Purpose and Environment	3531

A.9.1.53.2	Test parameters	3531

A.9.1.63.3	Test Requirements	3534

A.9.1.64	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	3534

A.9.1.64.1	Test Purpose and Environment	3534

A.9.1.64.2	Test parameters	3534

A.9.1.64.3	Test Requirements	3535

A.9.1.65	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	3536

A.9.1.65.1	Test Purpose and Environment	3536

A.9.1.65.2	Test parameters	3536

A.9.1.65.3	Test Requirements	3542

A.9.1.66	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	3542

A.9.1.66.1	Test Purpose and Environment	3542

A.9.1.66.2	Test parameters	3542

A.9.1.66.3	Test Requirements	3548

A.9.1.67	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	3548

A.9.1.67.1	Test Purpose and Environment	3548

A.9.1.67.2	Test parameters	3548

A.9.1.67.3	Test Requirements	3549

A.9.1.68	3 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3550

A.9.1.68.1	Test Purpose and Environment	3550

A.9.1.68.2	Test parameters	3550

A.9.1.68.3	Test Requirements	3554

A.9.1.69	4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3554

A.9.1.69.1	Test Purpose and Environment	3554

A.9.1.69.2	Test parameters	3554

A.9.1.69.3	Test Requirements	3558

A.9.1.70	5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3559

A.9.1.70.1	Test Purpose and Environment	3559

A.9.1.70.2	Test parameters	3559

A.9.1.70.3	Test Requirements	3571

A.9.1.71	6 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3572

A.9.1.71.1	Test Purpose and Environment	3572

A.9.1.71.2	Test parameters	3572

A.9.1.71.3	Test Requirements	3584

A.9.1.72	7 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3585

A.9.1.72.1	Test Purpose and Environment	3585

A.9.1.72.2	Test parameters	3585

A.9.1.72.3	Test Requirements	3597

A.9.1.73	FDD Intra frequency case for CA Idle Mode Measurements	3598

A.9.1.73.1	Test Purpose and Environment	3598

A.9.1.73.2	Test parameters	3598

A.9.1.73.3	Test Requirements	3601

A.9.1.74	FDD—FDD Inter frequency case for CA Idle Mode Measurements for overlapping carrier	3601

A.9.1.74.1	Test Purpose and Environment	3601

A.9.1.74.2	Test parameters	3601

A.9.1.74.3	Test Requirements	3602

A.9.1.75	FDD—FDD Inter frequency case for CA Idle Mode Measurements for non-overlapping carrier	3602

A.9.1.75.1	Test Purpose and Environment	3602

A.9.1.75.2	Test parameters	3603

A.9.1.75.3	Test Requirements	3604

A.9.1.76	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	3604

A.9.1.76.1	Test Purpose and Environment	3604

A.9.1.76.2	Test parameters	3604

A.9.1.76.3	Test Requirements	3607

A.9.1.77	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	3607

A.9.1.77.1	Test Purpose and Environment	3607

A.9.1.77.2	Test parameters	3607

A.9.1.77.3	Test Requirements	3610

A.9.1.78	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	3610

A.9.1.78.1	Test Purpose and Environment	3610

A.9.1.78.2	Test parameters	3610

A.9.1.78.3	Test Requirements	3613

A.9.1.79	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	3613

A.9.1.79.1	Test Purpose and Environment	3613

A.9.1.79.2	Test parameters	3613

A.9.1.79.3	Test Requirements	3616

A.9.1.80	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	3616

A.9.1.80.1	Test Purpose and Environment	3616

A.9.1.80.2	Test parameters	3616

A.9.1.80.3	Test Requirements	3619

A.9.1.81	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	3619

A.9.1.81.1	Test Purpose and Environment	3619

A.9.1.78.2	Test parameters	3619

A.9.1.81.3	Test Requirements	3622

A.9.2	RSRQ	3622

A.9.2.1	FDD Intra frequency case	3622

A.9.2.1.1	Test Purpose and Environment	3622

A.9.2.1.2	Test parameters	3622

A.9.2.1.3	Test Requirements	3625

A.9.2.2	TDD Intra frequency case	3625

A.9.2.2.1	Test Purpose and Environment	3625

A.9.2.2.2	Test parameters	3625

A.9.2.2.3	Test Requirements	3627

A.9.2.3	FDD—FDD Inter frequency case	3627

A.9.2.3.1	Test Purpose and Environment	3627

A.9.2.3.2	Test parameters	3627

A.9.2.3.3	Test Requirements	3630

A.9.2.4	TDD—TDD Inter frequency case	3630

A.9.2.4.1	Test Purpose and Environment	3630

A.9.2.4.2	Test parameters	3630

A.9.2.4.3	Test Requirements	3632

A.9.2.4A	FDD—TDD Inter frequency case	3633

A.9.2.4A.1	Test Purpose and Environment	3633

A.9.2.4A.2	Test parameters	3633

A.9.2.4A.3	Test Requirements	3635

A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation	3635

A.9.2.5.1	Test Purpose and Environment	3635

A.9.2.5.2	Test parameters	3636

A.9.2.5.3	Test Requirements	3639

A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation	3639

A.9.2.6.1	Test Purpose and Environment	3639

A.9.2.6.2	Test parameters	3639

A.9.2.6.3	Test Requirements	3642

A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	3642

A.9.2.7.1	Test Purpose and Environment	3642

A.9.2.7.2	Test parameters	3642

A.9.2.7.3	Test Requirements	3646

A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	3646

A.9.2.8.1	Test Purpose and Environment	3646

A.9.2.8.2	Test parameters	3646

A.9.2.8.3	Test Requirements	3650

A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS	3650

A.9.2.9.1	Test Purpose and Environment	3650

A.9.2.9.2	Test parameters	3650

A.9.2.9.3	Test Requirements	3654

A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS	3654

A.9.2.10.1	Test Purpose and Environment	3654

A.9.2.10.2	Test parameters	3654

A.9.2.10.3	Test Requirements	3658

A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	3658

A.9.2.11.1	Test Purpose and Environment	3658

A.9.2.11.2	Test parameters	3658

A.9.2.11.3	Test Requirements	3659

A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	3659

A.9.2.12.1	Test Purpose and Environment	3659

A.9.2.12.2	Test parameters	3660

A.9.2.12.3	Test Requirements	3660

A.9.2.13	Void	3661

A.9.2.13.1	Void	3661

A.9.2.13.2	Void	3661

A.9.2.13.3	Void	3661

A.9.2.14	Void	3661

A.9.2.14.1	Void	3661

A.9.2.14.2	Void	3661

A.9.2.14.3	Void	3661

A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3661

A.9.2.15.1	Test Purpose and Environment	3661

A.9.2.15.2	Test parameters	3661

A.9.2.15.3	Test Requirements	3665

A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3665

A.9.2.16.1	Test Purpose and Environment	3665

A.9.2.16.2	Test parameters	3665

A.9.2.16.3	Test Requirements	3670

A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth	3670

A.9.2.17.1	Test Purpose and Environment	3670

A.9.2.17.2	Test parameters	3670

A.9.2.17.3	Test Requirements	3672

A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth	3672

A.9.2.18.1	Test Purpose and Environment	3672

A.9.2.18.2	Test parameters	3672

A.9.2.18.3	Test Requirements	3675

A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ	3675

A.9.2.19.1	Test Purpose and Environment	3675

A.9.2.19.2	Test parameters	3675

A.9.2.19.3	Test Requirements	3677

A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ	3677

A.9.2.20.1	Test Purpose and Environment	3677

A.9.2.20.2	Test parameters	3677

A.9.2.20.3	Test Requirements	3680

A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	3680

A.9.2.21.1	Test Purpose and Environment	3680

A.9.2.21.2	Test parameters	3680

A.9.2.21.3	Test Requirements	3683

A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	3683

A.9.2.22.1	Test Purpose and Environment	3684

A.9.2.22.2	Test parameters	3684

A.9.2.22.3	Test Requirements	3684

A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	3684

A.9.2.23.1	Test Purpose and Environment	3684

A.9.2.23.2	Test parameters	3684

A.9.2.23.3	Test Requirements	3687

A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	3687

A.9.2.24.1	Test Purpose and Environment	3687

A.9.2.24.2	Test parameters	3687

A.9.2.24.3	Test Requirements	3688

A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	3688

A.9.2.25.1	Test Purpose and Environment	3688

A.9.2.25.2	Test parameters	3688

A.9.2.25.3	Test Requirements	3693

A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	3693

A.9.2.26.1	Test Purpose and Environment	3693

A.9.2.26.2	Test parameters	3693

A.9.2.26.3	Test Requirements	3697

A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz	3697

A.9.2.27.1	Test Purpose and Environment	3697

A.9.2.27.2	Test parameters	3697

A.9.2.27.3	Test Requirements	3698

A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	3698

A.9.2.28.1	Test Purpose and Environment	3698

A.9.2.28.2	Test parameters	3698

A.9.2.28.3	Test Requirements	3701

A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	3701

A.9.2.29.1	Test Purpose and Environment	3701

A.9.2.29.2	Test parameters	3701

A.9.2.29.3	Test Requirements	3704

A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	3704

A.9.2.30.1	Test Purpose and Environment	3704

A.9.2.30.2	Test parameters	3704

A.9.2.30.3	Test Requirements	3707

A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	3707

A.9.2.31.1	Test Purpose and Environment	3707

A.9.2.31.2	Test parameters	3707

A.9.2.31.3	Test Requirements	3710

A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	3710

A.9.2.32.1	Test Purpose and Environment	3710

A.9.2.32.2	Test parameters	3710

A.9.2.32.3	Test Requirements	3713

A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	3713

A.9.2.33.1	Test Purpose and Environment	3713

A.9.2.33.2	Test parameters	3713

A.9.2.33.3	Test Requirements	3717

A.9.2.34	FDD—FDD Inter frequency new RSRQ	3717

A.9.2.34.1	Test Purpose and Environment	3717

A.9.2.34.2	Test parameters	3717

A.9.2.34.3	Test Requirements	3720

A.9.2.35	TDD—TDD Inter frequency new RSRQ	3720

A.9.2.35.1	Test Purpose and Environment	3720

A.9.2.35.2	Test parameters	3720

A.9.2.35.3	Test Requirements	3723

A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols	3723

A.9.2.36.1	Test Purpose and Environment	3723

A.9.2.36.2	Test parameters	3723

A.9.2.36.3	Test Requirements	3724

A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols	3725

A.9.2.37.1	Test Purpose and Environment	3725

A.9.2.37.2	Test parameters	3725

A.9.2.37.3	Test Requirements	3726

A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	3727

A.9.2.38.1	Test Purpose and Environment	3727

A.9.2.38.2	Test parameters	3727

A.9.2.38.3	Test Requirements	3731

A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	3731

A.9.2.39.1	Test Purpose and Environment	3731

A.9.2.39.2	Test parameters	3731

A.9.2.39.3	Test Requirements	3735

A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	3735

A.9.2.40.1	Test Purpose and Environment	3735

A.9.2.40.2	Test parameters	3735

A.9.2.40.3	Test Requirements	3738

A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	3739

A.9.2.41.1	Test Purpose and Environment	3739

A.9.2.41.2	Test parameters	3739

A.9.2.41.3	Test Requirements	3742

A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0	3742

A.9.2.42.1	Test Purpose and Environment	3742

A.9.2.42.2	Test parameters	3742

A.9.2.42.3	Test Requirements	3745

A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0	3745

A.9.2.43.1	Test Purpose and Environment	3745

A.9.2.43.2	Test parameters	3745

A.9.2.43.3	Test Requirements	3748

A.9.2.44	TDD RSRQ Intra frequency case for UE category 0	3748

A.9.2.44.1	Test Purpose and Environment	3748

A.9.2.44.2	Test parameters	3748

A.9.2.44.3	Test Requirements	3750

A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation	3750

A.9.2.45.1	Test Purpose and Environment	3750

A.9.2.45.2	Test parameters	3750

A.9.2.45.3	Test Requirements	3754

A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation	3755

A.9.2.46.1	Test Purpose and Environment	3755

A.9.2.46.2	Test parameters	3755

A.9.2.46.3	Test Requirements	3759

A.9.2.47	5 DL FDD-TDD with PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	3760

A.9.2.47.1	Test Purpose and Environment	3760

A.9.2.47.2	Test parameters	3760

A.9.2.47.3	Test Requirements	3764

A.9.2.48	5 DL FDD-TDD with PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	3764

A.9.2.48.1	Test Purpose and Environment	3764

A.9.2.48.2	Test parameters	3765

A.9.2.48.3	Test Requirements	3769

A.9.2.49	5 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	3770

A.9.2.49.1	Test Purpose and Environment	3770

A.9.2.49.2	Test parameters	3770

A.9.2.49.3	Test Requirements	3776

A.9.2.50	5 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	3777

A.9.2.50.1	Test Purpose and Environment	3777

A.9.2.50.2	Test parameters	3777

A.9.2.50.3	Test Requirements	3783

A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell	3783

A.9.2.51.1	Test Purpose and Environment	3783

A.9.2.51.2	Test parameters	3784

A.9.2.51.3	Test Requirements	3788

A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell	3788

A.9.2.52.1	Test Purpose and Environment	3788

A.9.2.52.2	Test parameters	3788

A.9.2.52.3	Test Requirements	3791

A.9.2.53	4DL FDD RSRQ for E-UTRAN in Carrier Aggregation	3792

A.9.2.53.1	Test Purpose and Environment	3792

A.9.2.53.2	Test parameters	3792

A.9.2.53.3	Test Requirements	3798

A.9.2.54	4DL TDD RSRQ for E-UTRAN in Carrier Aggregation	3799

A.9.2.54.1	Test Purpose and Environment	3799

A.9.2.54.2	Test parameters	3799

A.9.2.54.3	Test Requirements	3805

A.9.2.55	3 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3805

A.9.2.55.1	Test Purpose and Environment	3805

A.9.2.55.2	Test parameters	3806

A.9.2.55.3	Test Requirements	3811

A.9.2.56	4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3811

A.9.2.56.1	Test Purpose and Environment	3811

A.9.2.56.2	Test parameters	3811

A.9.2.56.3	Test Requirements	3816

A.9.2.57	5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3816

A.9.2.57.1	Test Purpose and Environment	3816

A.9.2.57.2	Test parameters	3817

A.9.2.57.3	Test Requirements	3821

A.9.2.58	6 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3822

A.9.2.58.1	Test Purpose and Environment	3822

A.9.2.58.2	Test parameters	3822

A.9.2.58.3	Test Requirements	3826

A.9.2.59	7 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3827

A.9.2.59.1	Test Purpose and Environment	3827

A.9.2.59.2	Test parameters	3827

A.9.2.59.3	Test Requirements	3832

A.9.2.60	FDD Intra frequency case for CA Idle Mode Measurements	3833

A.9.2.60.1	Test Purpose and Environment	3833

A.9.2.60.2	Test parameters	3833

A.9.2.60.3	Test Requirements	3836

A.9.2.61	FDD—FDD Inter frequency case for CA Idle Mode Measurements on overlapping carrier	3836

A.9.2.61.1	Test Purpose and Environment	3836

A.9.2.61.2	Test parameters	3836

A.9.2.61.3	Test Requirements	3839

A.9.2.62	FDD—FDD Inter frequency case for CA Idle Mode Measurements on non-overlapping carrier	3839

A.9.2.62.1	Test Purpose and Environment	3839

A.9.2.62.2	Test parameters	3839

A.9.2.62.3	Test Requirements	3842

A.9.3	UTRAN FDD CPICH RSCP	3842

A.9.3.1	E-UTRAN FDD	3842

A.9.3.1.1	Test Purpose and Environment	3842

A.9.3.1.2	Parameters	3842

A.9.3.1.3	Test Requirements	3845

A.9.3.2 E-UTRAN TDD	3845

A.9.3.2.1	Test Purpose and Environment	3845

A.9.3.2.2	Parameters	3845

A.9.3.2.3	Test Requirements	3848

A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth	3848

A.9.3.3.1	Test Purpose and Environment	3848

A.9.3.3.2	Parameters	3848

A.9.3.3.3	Test Requirements	3849

A.9.4	UTRAN FDD CPICH Ec/No	3850

A.9.4.1	E-UTRAN FDD	3850

A.9.4.1.1	Test Purpose and Environment	3850

A.9.4.1.2	Parameters	3850

A.9.4.1.3	Test Requirements	3852

A.9.4.2	E-UTRAN TDD	3853

A.9.4.2.1	Test Purpose and Environment	3853

A.9.4.2.2	Parameters	3853

A.9.4.2.3	Test Requirements	3855

A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth	3856

A.9.4.3.1	Test Purpose and Environment	3856

A.9.4.3.2	Parameters	3856

A.9.4.3.3	Test Requirements	3857

A.9.5	UTRAN TDD measurement	3857

A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD	3857

A.9.5.1.1	Test Purpose and Environment	3857

A.9.5.1.2	Test parameters	3857

A.9.5.1.3	Test Requirements	3859

A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD	3859

A.9.5.2.1	Test Purpose and Environment	3859

A.9.5.2.2	Test parameters	3859

A.9.5.2.3	Test Requirements	3861

A.9.6	GSM Carrier RSSI	3862

A.9.6.1	E-UTRAN FDD	3862

A.9.6.1.1	Test Purpose and Environment	3862

A.9.6.1.2	Test Requirements	3863

A.9.6.2	E-UTRAN TDD	3863

A.9.6.2.1	Test Purpose and Environment	3863

A.9.6.2.2	Test Requirements	3865

A.9.7	UE Rx – Tx Time Difference	3866

A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case	3866

A.9.7.1.1	Test Purpose and Environment	3866

A.9.7.1.2	Test parameters	3866

A.9.7.1.3	Test Requirements	3867

A.9.7.2	E-UTRA TDD UE Rx – Tx time difference case	3867

A.9.7.2.1	Test Purpose and Environment	3867

A.9.7.2.2	Test parameters	3867

A.9.7.2.3	Test Requirements	3869

A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3869

A.9.7.3.1	Test Purpose and Environment	3869

A.9.7.3.2	Test parameters	3869

A.9.7.3.3	Test Requirements	3872

A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3872

A.9.7.4.1	Test Purpose and Environment	3872

A.9.7.4.2	Test Parameters	3872

A.9.7.4.3	Test Requirements	3875

A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3875

A.9.7.5.1	Test Purpose and Environment	3875

A.9.7.5.2	Test parameters	3875

A.9.7.5.3	Test Requirements	3878

A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3878

A.9.7.6.1	Test Purpose and Environment	3878

A.9.7.6.2	Test Parameters	3878

A.9.7.6.3	Test Requirements	3881

A.9.7.7	E-UTRAN FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	3881

A.9.7.7.1	Test Purpose and Environment	3881

A.9.7.7.2	Test parameters	3881

A.9.7.7.3	Test Requirements	3882

A.9.7.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	3883

A.9.7.8.1	Test Purpose and Environment	3883

A.9.7.8.2	Test parameters	3883

A.9.7.8.3	Test Requirements	3884

A.9.7.9	E-UTRAN TDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	3884

A.9.7.9.1	Test Purpose and Environment	3884

A.9.7.9.2	Test parameters	3884

A.9.7.9.3	Test Requirements	3885

A.9.8	RSTD	3886

A.9.8.1	E-UTRAN FDD RSTD intra frequency case	3886

A.9.8.1.1	Test Purpose and Environment	3886

A.9.8.1.2	Test Requirements	3888

A.9.8.1.2A	Test Requirements for UE Category 1bis	3888

A.9.8.2	E-UTRAN TDD RSTD intra frequency case	3889

A.9.8.2.1	Test Purpose and Environment	3889

A.9.8.2.2	Test Requirements	3892

A.9.8.2.2A	Test Requirements for UE Category 1bis	3892

A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case	3893

A.9.8.3.1	Test Purpose and Environment	3893

A.9.8.3.2	Test Requirements	3895

A.9.8.3.2A	Test Requirements for UE Category 1bis	3895

A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case	3896

A.9.8.4.1	Test Purpose and Environment	3896

A.9.8.4.2	Test Requirements	3898

A.9.8.4.2A	Test Requirements for UE Category 1bis	3898

A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation	3899

A.9.8.5.1	Test Purpose and Environment	3899

A.9.8.5.2	Test Requirements	3901

A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation	3901

A.9.8.6.1	Test Purpose and Environment	3901

A.9.8.6.2	Test Requirements	3904

A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	3904

A.9.8.7.1	Test Purpose and Environment	3904

A.9.8.7.2	Test Requirements	3905

A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	3905

A.9.8.8.1	Test Purpose and Environment	3905

A.9.8.8.2	Test Requirements	3906

A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	3906

A.9.8.9.1	Test Purpose and Environment	3906

A.9.8.9.2	Test Requirements	3907

A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	3907

A.9.8.10.1	Test Purpose and Environment	3907

A.9.8.10.2	Test Requirements	3908

A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth	3908

A.9.8.11.1	Test Purpose and Environment	3908

A.9.8.11.2	Test Requirements	3909

A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth	3909

A.9.8.12.1	Test Purpose and Environment	3909

A.9.8.12.2	Test Requirements	3910

A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz	3910

A.9.8.13.1	Test Purpose and Environment	3910

A.9.8.13.2	Test Requirements	3911

A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	3911

A.9.8.14.1	Test Purpose and Environment	3911

A.9.8.14.2	Test Requirements	3918

A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	3918

A.9.8.15.1	Test Purpose and Environment	3918

A.9.8.15.2	Test Requirements	3924

A.9.8.16	HD – FDD Intra frequency case for UE Category NB1 inband mode in normal coverage	3924

A.9.8.16.1	Test Purpose and Environment	3924

A.9.8.16.2	Test Requirements	3927

A.9.8.17	HD – FDD Inter frequency case for UE Category NB1 inband mode in normal coverage	3927

A.9.8.17.1	Test Purpose and Environment	3927

A.9.8.17.2	Test Requirements	3930

A.9.8.18	HD – FDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	3930

A.9.8.18.1	Test Purpose and Environment	3930

A.9.8.18.2	Test Requirements	3933

A.9.8.19	HD – FDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	3933

A.9.8.19.1	Test Purpose and Environment	3933

A.9.8.19.2	Test Requirements	3936

A.9.8.20	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode A	3936

A.9.8.20.1	Test Purpose and Environment	3936

A.9.8.20.2	Test Requirements	3939

A.9.8.21	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CEModeA	3940

A.9.8.21.1	Test Purpose and Environment	3940

A.9.8.21.2	Test Requirements	3943

A.9.8.22	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode A	3944

A.9.8.22.1	Test Purpose and Environment	3944

A.9.8.22.2	Test Requirements	3947

A.9.8.23	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode B	3948

A.9.8.23.1	Test Purpose and Environment	3948

A.9.8.23.2	Test Requirements	3951

A.9.8.24	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CE Mode B	3951

A.9.8.24.1	Test Purpose and Environment	3951

A.9.8.24.2	Test Requirements	3955

A.9.8.25	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode B	3956

A.9.8.25.1	Test Purpose and Environment	3956

A.9.8.25.2	Test Requirements	3959

A.9.8.26	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	3960

A.9.8.26.1	Test Purpose and Environment	3960

A.9.8.26.2	Test Requirements	3962

A.9.8.27	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	3962

A.9.8.27.1	Test Purpose and Environment	3962

A.9.8.27.2	Test Requirements	3965

A.9.8.28	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode A	3965

A.9.8.28.1	Test Purpose and Environment	3965

A.9.8.28.2	Test Requirements	3968

A.9.8.29	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	3968

A.9.8.29.1	Test Purpose and Environment	3968

A.9.8.29.2	Test Requirements	3972

A.9.8.30	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	3973

A.9.8.30.1	Test Purpose and Environment	3973

A.9.8.30.2	Test Requirements	3976

A.9.8.31	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode B	3977

A.9.8.31.1	Test Purpose and Environment	3977

A.9.8.31.2	Test Requirements	3981

A.9.8.32	TDD Intra frequency case for UE Category NB1 inband mode in normal coverage	3981

A.9.8.32.1	Test Purpose and Environment	3981

A.9.8.32.2	Test Requirements	3984

A.9.8.33	TDD Inter frequency case for UE Category NB1 inband mode in normal coverage	3984

A.9.8.33.1	Test Purpose and Environment	3984

A.9.8.33.2	Test Requirements	3987

A.9.8.34	TDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	3987

A.9.8.34.1	Test Purpose and Environment	3987

A.9.8.34.2	Test Requirements	3990

A.9.8.35	TDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	3990

A.9.8.35.1	Test Purpose and Environment	3990

A.9.8.35.2	Test Requirements	3993

A.9.9	RSRP and RSRQ on the serving cell	3993

A.9.9.1	FDD Intra frequency serving cell case	3993

A.9.9.1.1	Test Purpose and Environment	3993

A.9.9.1.2	Test parameters	3993

A.9.9.1.3	Test Requirements	3996

A.9.9.2	TDD Intra frequency serving cell case	3996

A.9.9.2.1	Test Purpose and Environment	3996

A.9.9.2.2	Test parameters	3996

A.9.9.2.3	Test Requirements	3998

A.9.10	SSTD	3998

A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC	3998

A.9.10.1.1	Test Purpose and Environment	3998

A.9.10.1.2	Test parameters	3998

A.9.10.1.3	Test Requirements	4000

A.9.10.2	Void	4000

A.9.10.3	Void	4000

A.9.10.4	Void	4000

A.9.11 RSSI	4000

A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)	4000

A.9.11.1.1	Test Purpose and Environment	4000

A.9.11.1.2	Test parameters	4000

A.9.11.1.3	Test Requirements	4003

A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)	4003

A.9.11.2.1	Test Purpose and Environment	4003

A.9.11.2.2	Test parameters	4003

A.9.12	Channel occupancy	4006

A.9.12.1	FS3 channel occupancy test (PCell using FDD)	4006

A.9.12.1.1	Test Purpose and Environment	4006

A.9.12.1.2	Test parameters	4006

A.9.12.1.3	Test Requirements	4009

A.9.12.2	FS3 channel occupancy test (PCell using TDD)	4009

A.9.12.2.1	Test Purpose and Environment	4009

A.9.12.2.2	Test parameters	4009

A.9.12.2.3	Test Requirements	4012

A.9.13	RS-SINR	4012

A.9.13.1	FDD Intra-Frequency Case	4012

A.9.13.1.1	Test Purpose and Environment	4012

A.9.13.1.2	Test parameters	4012

A.9.13.1.3	Test Requirements	4015

A.9.13.2	TDD Intra-Frequency Case	4015

A.9.13.2.1	Test Purpose and Environment	4015

A.9.13.2.2	Test parameters	4015

A.9.13.2.3	Test Requirements	4018

A.9.13.3	FDD—FDD Inter frequency case	4018

A.9.13.3.1	Test Purpose and Environment	4018

A.9.13.3.2	Test parameters	4018

A.9.13.3.3	Test Requirements	4021

A.9.13.4	TDD—TDD Inter frequency case	4021

A.9.13.4.1	Test Purpose and Environment	4021

A.9.13.4.2	Test parameters	4021

A.9.13.4.3	Test Requirements	4025

A.9.13.5	FDD—TDD Inter frequency case	4026

A.9.13.5.1	Test Purpose and Environment	4026

A.9.13.5.2	Test parameters	4026

A.9.13.5.3	Test Requirements	4032

A.9.13.6	TDD—FDD Inter frequency case	4032

A.9.13.6.1	Test Purpose and Environment	4032

A.9.13.6.2	Test parameters	4032

A.9.13.6.3	Test Requirements	4036

A.9.14	Channel quality reporting accuracy	4036

A.9.14.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	4036

A.9.14.1.1	Test Purpose and Environment	4036

A.9.14.1.2	Test parameters	4037

A.9.14.1.3	Test Requirements	4037

A.9.14.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	4038

A.9.14.2.1	Test Purpose and Environment	4038

A.9.14.2.2	Test parameters	4038

A.9.14.2.3	Test Requirements	4039

A.9.14.3	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage	4039

A.9.14.3.1	Test Purpose and Environment	4039

A.9.14.3.2	Test parameters	4039

A.9.14.3.3	Test Requirements	4040

A.9.14.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage	4040

A.9.14.4.1	Test Purpose and Environment	4040

A.9.14.4.2	Test parameters	4040

A.9.14.4.3	Test Requirements	4041

A.10	Proximity-based Services in Any Cell Selection State	4064

A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	4064

A.10.1.1	Test Purpose and Environment	4064

A.10.1.2	Test Requirements	4065

A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication	4066

A.10.2.1	Test Purpose and Environment	4066

A.10.2.2	Test Requirements	4067

A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test	4067

A.10.3.1	Test Purpose and Environment	4067

A.10.3.2	Test Requirements	4069

A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)	4070

A.10.4.1	Test Purpose and Environment	4070

A.10.4.2	Test Requirements	4072

A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier	4072

A.11.1	V2V UE Transmission Timing Accuracy Test	4072

A.11.1.1	Test Purpose and Environment	4072

A.11.1.2	Test requirements	4073

A.11.2	Interruptions due to V2V sidelink communication	4073

A.11.2.1	Test Purpose and Environment	4073

A.11.2.2	Test Requirements	4075

A.12	4075

A.12.1	V2X UE Transmission Timing Accuracy Test	4075

A.12.1.1	V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference	4075

A.12.1.1.1	Test Purpose and Environment	4075

A.12.1.1.2	Test requirements	4076

A.12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference	4077

A.12.1.2.1	Test Purpose and Environment	4077

A.12.1.2.2	Test Requirements	4077

A.12.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	4078

A.12.2.1	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference	4078

A.12.2.1.1	Test Purpose and Environment	4078

A.12.2.1.2	Test Requirements	4079

A.12.2.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference	4080

A.12.2.2.1	Test Purpose and Environment	4080

A.12.2.2.2	Test Requirements	4081

A.12.3	V2X Synchronization Reference Selection/Reselection Tests	4081

A.12.3.1	V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority	4081

A.12.3.1.1	Test Purpose and Environment	4081

A.12.3.1.2	Test Requirements	4084

A.12.3.2	V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority	4085

A.12.3.2.1	Test Purpose and Environment	4085

A.12.3.1.2	Test Requirements	4087

A.12.4	Congestion Control Measurement Test for V2X UE	4087

A.12.4.1	Test Purpose and Environment	4087

A.12.4.2	Test Requirements	4089

A.12.5	Interruptions due to V2X Sidelink Communication	4089

A.12.5.1	Test Purpose and Environment	4089

A.12.5.2	Test Requirements	4091

A.12.6	V2X UE Autonomous Resource Selection/Reselection Measurement Test	4091

A.12.6.1	V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements	4091

A.12.6.1.1	Test Purpose and Environment	4091

A.12.6.1.2	Test Requirements	4093

A.12.6.2	V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements	4093

A.12.6.2.1	Test Purpose and Environment	4093

A.12.6.1.2	Test Requirements	4095

A.12.7	V2X Synchronization Reference Selection/Reselection Tests for V2X Carrier Aggregation	4095

A.12.7.1	Test Purpose and Environment	4095

A.12.7.2	Test Requirements	4098

A.12.8	Interruptions due to V2X Carrier Aggregation	4099

A.12.8.1	Interruptions on a FDD PCell	4099

A.12.8.1.1	Test Purpose and Environment	4099

A.12.8.1.2	Test Requirements	4100

A.12.8.2	Interruptions on a TDD PCell	4100

A.12.8.2.1	Test Purpose and Environment	4100

A.12.8.2.2	Test Requirements	4102

A.13	E-UTRAN Standalone Tests for UE Category NB for Satellite Access	4103

A.13.1	RRC_IDLE state for satellite access	4103

A.13.1.1	Cell re-selection for satellite access	4103

A.13.1.1.1	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage	4103

A.13.1.1.1.1	Test Purpose and Environment	4103

A.13.1.1.1.2	Test Requirements	4105

A.13.1.1.2	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with serving cell RRM measurement relaxation	4106

A.13.1.1.2.1	Test Purpose and Environment	4106

A.13.1.1.3	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX	4109

A.13.1.1.3.1	Test Purpose and Environment	4109

A.13.1.1.3.2	Test Requirements	4111

A.13.1.1.4	HD – FDD Inter frequency case for UE Category NB1 Standalone mode in normal coverage	4112

A.13.1.1.4.1	Test Purpose and Environment	4112

A.13.1.1.4.2	Test Requirements	4113

A.13.1.1.5	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in enhanced coverage, location-based cell reselection for NGSO	4114

A.13.1.1.5.1	Test Purpose and Environment	4114

A.13.1.1.5.2	Test Requirements	4115

A.13.1.1.6	HD – FDD Inter frequency case for UE Category NB1 Standalone mode in enhanced coverage, time-based cell reselection for NGSO	4116

A.13.1.1.6.1	Test Purpose and Environment	4116

A.13.1.1.6.2	Test Requirements	4117

A.13.2	Void	4118

A.13.3	RRC connection mobility control for satellite access	4118

A.13.3.1	RRC re-establishment for satellite access	4118

A.13.3.1.1	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage	4118

A.13.3.1.1.1	Test Purpose and Environment	4118

A.13.3.1.1.2	Test Requirements	4119

A.13.3.1.2	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage	4120

A.13.3.1.2.1	Test Purpose and Environment	4120

A.13.3.1.2.2	Test Requirements	4121

A.13.3.1.3	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage	4122

A.13.3.1.3.1	Test Purpose and Environment	4122

A.13.3.1.3.2	Test Requirements	4123

A.13.3.2	Random Access for Satellite Access	4123

A.13.3.2.1	Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in normal coverage	4124

A.13.3.2.1.1	Test Purpose and Environment	4124

A.13.3.2.1.2	Test Requirements	4126

A.13.3.2.2	Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in Enhanced Coverage	4128

A.13.3.2.2.1	Test Purpose and Environment	4128

A.13.3.2.2.2	Test Requirements	4129

A.13.3.2.3	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs Standalone mode in Enhanced Coverage	4131

A.13.3.2.3.1	Test Purpose and Environment	4131

A.13.3.2.3.2	Test Requirements	4132

A.13.4	Timing and signalling characteristics for satellite access	4134

A.13.4.1	UE transmit timing for satellite access	4134

A.13.4.1.1	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under normal coverage for Satellite Access	4134

A.13.4.1.1.1	Test Purpose and Environment	4134

A.13.4.1.1.2	Test Requirements	4135

A.13.4.1.2	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage for Satellite Access	4136

A.13.4.1.2.1	Test Purpose and Environment	4136

A.13.4.1.2.2	Test Requirements	4137

A.13.4.1.3	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage with segment transmission in NGSO for Satellite Access	4138

A.13.4.1.3.1	Test Purpose and Environment	4138

A.13.4.1.3.2	Test Requirements	4139

A.13.4.2	UE timing advance for satellite access	4140

A.13.4.2.1	HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage for Satellite Access	4140

A.13.4.2.1.1	Test Purpose and Environment	4140

A.13.4.2.1.2	Test Requirements	4142

A.13.4.2.2	HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhance Coverage for Satellite Access	4142

A.13.4.2.2.1	Test Purpose and Environment	4142

A.13.4.2.2.2	Test Requirements	4144

A.13.4.3	Radio Link Monitoring for satellite access	4144

A.13.4.3.1	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in normal coverage	4144

A.13.4.3.1.1	Test Purpose and Environment	4144

A.13.4.3.1.2	Test Requirements	4147

A.13.4.3.2	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in enhanced coverage	4147

A.13.4.3.2.1	Test Purpose and Environment	4147

A.13.4.3.2.2	Test Requirements	4150

A.13.4.3.3	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 Standalone mode in Enhanced Coverage	4150

A.13.4.3.3.1	Test Purpose and Environment	4150

A.13.4.3.3.2	Test Requirements	4153

A.13.4.3.4	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 Standalone mode in Normal Coverage	4153

A.13.4.3.4.1	Test Purpose and Environment	4153

A.13.4.3.4.2	Test Requirements	4156

A.13.4.3.5	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	4156

A.13.4.3.5.1	Test Purpose and Environment	4156

A.13.4.3.5.2	Test Requirements	4159

A.13.4.3.6	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 Standalone mode in Enhanced Coverage	4159

A.13.4.3.6.1	Test Purpose and Environment	4159

A.13.4.3.6.2	Test Requirements	4162

A.13.4.3.7	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	4162

A.13.4.3.7.1	Test Purpose and Environment	4162

A.13.4.3.7.2	Test Requirements	4164

A.13.4.3.8	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Enhanced Coverage	4165

A.13.4.3.8.1	Test Purpose and Environment	4165

A.13.4.3.8.2	Test Requirements	4167

A.13.5	UE measurement procedures in RRC_CONNECTED state for UE category NB1 for satellite access	4168

A.13.6	Measurement performance requirements for UE for satellite access	4175

A.13.6.1	Void	4175

A.13.6.2	Channel quality reporting accuracy for satellite access	4175

A.13.6.2.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	4175

A.13.6.2.1.1	Test Purpose and Environment	4175

A.13.6.2.1.2	Test parameters	4175

A.13.6.2.1.3	Test Requirements	4176

A.13.6.2.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	4176

A.13.6.2.2.1	Test Purpose and Environment	4176

A.13.6.2.2.2	Test parameters	4176

A.13.6.2.2.3	Test Requirements	4177

A.13.6.2.3	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage	4177

A.13.6.2.3.1	Test Purpose and Environment	4177

A.13.6.2.3.2	Test parameters	4178

A.13.6.2.3.3	Test Requirements	4178

A.13.6.2.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage	4179

A.13.6.2.4.1	Test Purpose and Environment	4179

A.13.6.2.4.2	Test parameters	4179

A.13.6.2.4.3	Test Requirements	4180

A.13.6.2.5	E-UTRAN HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under normal coverage	4180

A.13.6.2.5.1	Test Purpose and Environment	4180

A.13.6.2.5.2	Test parameters	4180

A.13.6.2.5.3	Test Requirements	4181

A.13.6.2.6	E-UTRAN HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under enhanced coverage	4182

A.13.6.2.6.1	Test Purpose and Environment	4182

A.13.6.2.6.2	Test parameters	4182

A.13.6.2.6.3	Test Requirements	4183

A.14	E-UTRAN Standalone Tests for UE Category M1 for Satellite Access	4183

A.14.1	RRC_IDLE state for satellite access	4183

A.14.1.1	Cell re-selection for satellite access	4183

A.14.1.1.1	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	4183

A.14.1.1.1.1	Test Purpose and Environment	4183

A.14.1.1.1.2	Test Requirements	4185

A.14.1.1.2	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	4186

A.14.1.1.2.1	Test Purpose and Environment	4186

A.14.1.1.2.2	Test Requirements	4188

A.14.1.1.3	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	4189

A.14.1.1.3.1	Test Purpose and Environment	4189

A.14.1.1.3.2	Test Requirements	4191

A.14.1.1.4	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	4192

A.14.1.1.4.1	Test Purpose and Environment	4192

A.14.1.1.4.2	Test Requirements	4194

A.14.1.1.5	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	4195

A.14.1.1.5.1	Test Purpose and Environment	4195

A.14.1.1.5.2	Test Requirements	4197

A.14.1.1.6	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	4198

A.14.1.1.6.1	Test Purpose and Environment	4198

A.14.1.1.6.2	Test Requirements	4200

A.14.1.1.7	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage, time-based triggering	4201

A.14.1.1.7.1	Test Purpose and Environment	4201

A.14.1.1.7.2	Test Requirements	4202

A.14.1.1.8 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage, time-based triggering	4203

A.14.1.1.8.1	Test Purpose and Environment	4203

A.14.1.1.8.2	Test Requirements	4204

A.14.1.1.9	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage, location-based triggering	4205

A.14.1.1.9.1	Test Purpose and Environment	4205

A.14.1.1.9.2	Test Requirements	4206

A.14.1.1.10	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage, location-based triggering	4207

A.14.1.1.10.1	Test Purpose and Environment	4207

A.14.1.1.10.2	Test Requirements	4208

A.14.2	RRC_CONNECTED state mobility for satellite access	4209

A.14.2.1	E-UTRAN handover for satellite access	4209

A.14.2.1.1	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	4209

A.14.2.1.1.1	Test Purpose and Environment	4209

A.14.2.1.1.2	Test Requirements	4211

A.14.2.1.2	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	4212

A.14.2.1.2.1	Test Purpose and Environment	4212

A.14.2.1.2.2	Test Requirements	4213

A.14.2.1.3	E-UTRAN FDD-FDD Intra frequency conditional handover for Cat-M1 UEs in CEModeA	4214

A.14.2.1.3.1	Test Purpose and Environment	4214

A.14.2.1.3.2	Test Requirements	4215

A.14.2.1.4	E-UTRAN HD-FDD Intra frequency conditional handover for Cat-M1 UEs in CEModeA	4216

A.14.2.1.4.1	Test Purpose and Environment	4216

A.14.2.1.4.2	Test Requirements	4217

A.14.2.1.5	E-UTRAN FDD Intra frequency handover for Cat-M1 UEs in CEModeA	4218

A.14.2.1.5.1	Test Purpose and Environment	4218

A.14.2.1.5.2	Test Requirements	4219

14.2.1.6	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	4220

A.14.2.1.6.1	Test Purpose and Environment	4220

A.14.2.1.6.2	Test Requirements	4221

A.14.2.1.7	E-UTRAN FD-FDD Inter frequency handover for Cat-M1 UEs in CEModeA	4222

A.14.2.1.7.1	Test Purpose and Environment	4222

A.14.2.1.7.2	Test Requirements	4223

A.14.2.1.8	E-UTRAN HD-FDD Inter frequency handover for Cat-M1 UEs in CEModeA	4224

A.14.2.1.8.1	Test Purpose and Environment	4224

A.14.2.1.8.2	Test Requirements	4225

A.14.2.1.9	E-UTRAN FDD Inter frequency handover for Cat-M1 UEs in CEModeB	4226

A.14.2.1.9.1	Test Purpose and Environment	4226

A.14.2.1.9.2	Test Requirements	4227

A.14.2.1.10	E-UTRAN HD-FDD Inter frequency handover for Cat-M1 UEs in CEModeB	4228

A.14.2.1.10.1	Test Purpose and Environment	4228

A.14.2.1.10.2	Test Requirements	4229

A.14.2.1.11	E-UTRAN FDD-FDD Inter frequency conditional handover for Cat-M1 UEs in CEModeA	4230

A.14.2.1.11.1	Test Purpose and Environment	4230

A.14.2.1.12.2	Test Requirements	4231

A.14.2.1.12	E-UTRAN HD-FDD Inter frequency conditional handover for Cat-M1 UEs in CEModeA	4232

A.14.2.1.12.1	Test Purpose and Environment	4232

A.14.2.1.12.2	Test Requirements	4233

A.14.2.1.15	E-UTRAN FDD-FDD Inter frequency location based conditional handover for Cat-M1 UEs in CEModeA	4238

A.14.2.1.15.1	Test Purpose and Environment	4238

A.14.2.1.16.2	Test Requirements	4239

A.14.2.1.16	E-UTRAN HD-FDD Inter frequency time based conditional handover for Cat-M1 UEs in CEModeA	4240

A.14.2.1.16.1	Test Purpose and Environment	4240

A.14.2.1.16.2	Test Requirements	4241

A.14.3	RRC connection mobility control for satellite access	4242

A.14.3.1	RRC re-establishment for satellite access	4242

A.14.3.1.1	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA for Satellite access	4242

A.14.3.1.1.1	Test Purpose and Environment	4242

A.14.3.1.1.2	Test Requirements	4243

A.14.3.1.2	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	4244

A.14.3.1.2.1	Test Purpose and Environment	4244

A.14.3.1.2.2	Test Requirements	4246

A.14.3.1.3	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA for Satellite access	4247

A.14.3.1.3.1	Test Purpose and Environment	4247

A.14.3.1.3.2	Test Requirements	4249

A.14.3.1.4	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA for Satellite access	4250

A.14.3.1.4.1	Test Purpose and Environment	4250

A.14.3.1.4.2	 Test Requirements	4252

A.14.3.2	Random access for satellite access	4253

A.14.3.2.1	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage for satellite access	4253

A.14.3.2.1.1	Test Purpose and Environment	4253

A.14.3.2.1.2	Test Requirements	4255

A.14.3.2.2	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage for satellite access	4256

A.14.3.2.2.1	Test Purpose and Environment	4256

A.14.3.2.2.2	Test Requirements	4258

A.14.3.2.3	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage for satellite access	4260

A.14.3.2.3.1	Test Purpose and Environment	4260

A.14.3.2.3.2	Test Requirements	4262

A.14.3.2.4	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage for satellite access	4263

A.14.3.2.4.1	Test Purpose and Environment	4263

A.14.3.2.4.2	Test Requirements	4265

A.14.4	Timing and signalling characteristics for satellite access	4267

A.14.4.1	UE transmit timing for satellite access	4267

A.14.4.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	4267

A.14.4.1.1.1	Test Purpose and Environment	4267

A.14.4.1.1.2	Test Requirements	4269

A.14.4.1.2	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	4270

A.14.4.1.2.1	Test Purpose and Environment	4270

A.14.4.1.2.2	Test Requirements	4271

A.14.4.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	4272

A.14.4.1.3.1	Test Purpose and Environment	4272

A.14.4.1.3.2	Test Requirements	4273

A.14.4.1.4	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	4274

A.14.4.1.4.1	Test Purpose and Environment	4274

A.14.4.1.4.2	Test Requirements	4275

A.14.4.1.5	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB with segment transmission in NGSO for Satellite Access	4276

A.14.4.1.5.1	Test Purpose and Environment	4276

A.14.4.1.5.2	Test Requirements	4277

A.14.4.2	UE timing advance for satellite access	4278

A.14.4.2.1	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	4278

A.14.4.2.1.1	Test Purpose and Environment	4278

A.14.4.2.1.2	Test Requirements	4280

A.14.4.2.2	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	4280

A.14.4.2.2.1	Test Purpose and Environment	4280

A.14.4.2.2.2	Test Requirements	4283

A.14.4.2.3	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	4283

A.14.4.2.3.1	Test Purpose and Environment	4283

A.14.4.2.3.2	Test Requirements	4284

A.14.4.2.4	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	4284

A.14.4.2.4.1	Test Purpose and Environment	4284

A.14.4.2.4.2	Test Requirements	4286

A.14.4.3	Radio Link Monitoring for satellite access	4287

A.14.4.3.1	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for Satellite access	4287

A.14.4.3.1.1	Test Purpose and Environment	4287

A.14.4.3.1.2	Test Requirements	4290

A.14.4.3.2	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A for Satellite access	4290

A.14.4.3.2.1	Test Purpose and Environment	4290

A.14.4.3.2.2	Test Requirements	4293

A.14.4.3.3	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for Satellite access	4293

A.14.4.3.3.1	Test Purpose and Environment	4293

A.14.4.3.3.2	Test Requirements	4296

A.14.4.3.4	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A for Satellite access	4296

A.14.4.3.4.1	Test Purpose and Environment	4296

A.14.4.3.4.2	Test Requirements	4299

A.14.4.3.5	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	4299

A.14.4.3.5.1	Test Purpose and Environment	4299

A.14.4.3.5.2	 Test Requirements	4302

A.14.4.3.6	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	4302

A.14.4.3.6.1	Test Purpose and Environment	4302

A.14.4.3.6.2	Test Requirements	4305

A.14.4.3.7	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	4305

A.14.4.3.7.1	Test Purpose and Environment	4305

A.14.4.3.8	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	4308

A.14.4.3.8.1 	Test Purpose and Environment	4308

A.14.4.3.8.2	Test Requirements	4311

A.14.5	UE measurement procedures in RRC_CONNECTED state for satellite access	4311

A.14.5.1 Intra-frequency measurements for satellite access	4311

A.14.5.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under AWGN conditions in asynchronous cells for Cat-M1 UE in CEModeA	4311

A.14.5.1.1.1	Test Purpose and Environment	4311

A.14.5.1.1.2	Test Requirements	4313

A.14.5.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under AWGN conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	4314

A.14.5.1.2.1	Test Purpose and Environment	4314

A.14.5.1.2.2	Test Requirements	4316

A.14.5.1.3	E-UTRAN HD-FDD intra-frequency event triggered reporting under AWGN conditions in asynchronous cells for Cat-M1 UE in CEModeA	4316

A.14.5.1.3.1	Test Purpose and Environment	4316

A.14.5.1.3.2	Test Requirements	4318

A.14.5.1.4	E-UTRAN HD-FDD intra-frequency event triggered reporting under AWGN conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	4319

A.14.5.1.4.1	Test Purpose and Environment	4319

A.14.5.1.4.2	Test Requirements	4321

A.14.5.1.5	E-UTRAN FD-FDD Intra-frequency event triggered reporting in asynchronous cells for UE category M1 in CEModeA with location-based triggering	4321

A.14.5.1.5.1	Test Purpose and Environment	4321

A.14.5.1.5.2	Test Requirements	4323

A.14.5.1.6 	E-UTRAN HD-FDD Intra-frequency event triggered reporting in asynchronous cells for UE category M1 in CEModeA with location-based triggering	4324

A.14.5.1.6.1	Test Purpose and Environment	4324

A.14.5.1.6.2	Test Requirements	4325

A.14.5.1.7	E-UTRAN HD-FDD Intra-frequency event triggered reporting in asynchronous cells for UE category M1 in CEModeA when DRX is used with time-based triggering	4326

A.14.5.1.7.1	Test Purpose and Environment	4326

A.14.5.1.7.2	Test Requirements	4327

A.14.5.2 Inter-frequency measurements for satellite access	4328

A.14.5.2.1	E-UTRAN FD-FDD Inter-frequency event triggered reporting in asynchronous cells for UE category M1 in CEModeA when DRX is used with time-based triggering	4328

A.14.5.2.1.1	Test Purpose and Environment	4328

A.14.5.2.1.2	Test Requirements	4329

A.14.5.2.2	E-UTRAN HD-FDD Inter-frequency event triggered reporting in asynchronous cells for UE category M1 in CEModeA when DRX is used with time-based triggering	4330

A.14.5.2.2.1	Test Purpose and Environment	4330

A.14.5.2.2.2	Test Requirements	4331

A.14.5.2.3	E-UTRAN HD-FDD Inter-frequency event triggered reporting in asynchronous cells for UE category M1 in CEModeB when DRX is used with time-based triggering	4332

A.14.5.2.3.1	Test Purpose and Environment	4332

A.14.5.2.3.2	Test Requirements	4333

A.14.5.2.4	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	4334

A.14.5.2.4.1	Test Purpose and Environment	4334

A.14.5.2.4.2	Test Requirement	4335

A.14.5.2.5	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	4336

A.14.5.2.5.1	Test Purpose and Environment	4336

A.14.5.2.5.2	Test Requirement	4338

A.14.5.2.6	E-UTRAN HD-FDD Inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	4338

A.14.5.2.6.1	Test Purpose and Environment	4338

A.14.5.2.6.2	Test Requirement	4340

A.14.5.2.7	E-UTRAN HD-FDD inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 in CEModeA in DRX	4340

A.14.5.2.7.1	Test Purpose and Environment	4340

A.14.5.2.7.2	Test Requirement	4342

A.14.5.2.8	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	4342

A.14.5.2.8.1	Test Purpose and Environment	4342

A.14.5.2.8.2	Test Requirement	4344

A.14.5.2.9	E-UTRAN HD-FDD inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	4345

A.14.5.2.9.1	Test Purpose and Environment	4345

A.14.5.2.9.2	Test Requirement	4346

A.14.6	Measurement performance requirements for UE for satellite access	4347

A.14.6.1	RSRP for satellite access	4347

A.14.6.1.1	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	4347

A.14.6.1.1.1	Test Purpose and Environment	4347

A.14.6.1.1.2	Test parameters	4347

A.14.6.1.1.3	Test Requirements	4349

A.14.6.1.2	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	4349

A.14.6.1.2.1	Test Purpose and Environment	4349

A.14.6.1.2.2	Test parameters	4349

A.14.6.1.2.3	Test Requirements	4351

A.14.6.1.3	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	4351

A.14.6.1.3.1	Test Purpose and Environment	4351

A.14.6.1.3.2	Test parameters	4351

A.14.6.1.3.3	Test Requirements	4352

A.14.6.1.4	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	4353

A.14.6.1.4.1	Test Purpose and Environment	4353

A.14.6.1.4.2	Test parameters	4353

A.14.6.1.4.3	Test Requirements	4354

A.14.6.1.5	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	4354

A.14.6.1.5.1	Test Purpose and Environment	4354

A.14.6.1.5.2	Test parameters	4354

A.14.6.1.5.3	Test Requirements	4355

A.14.6.1.6	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	4355

A.14.6.1.6.1	Test Purpose and Environment	4355

A.14.6.1.6.2	Test parameters	4356

A.14.6.1.6.3	Test Requirements	4357

A.14.6.2	Channel quality reporting accuracy for satellite access	4357

A.14.6.2.1	E-UTRAN FD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode A for Satellite access	4357

A.14.6.2.1.1	Test Purpose and Environment	4357

A.14.6.2.1.2	Test parameters	4357

A.14.6.2.1.3	Test Requirements	4358

A.14.6.2.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode A for Satellite access	4359

A.14.6.2.2.1	Test Purpose and Environment	4359

A.14.6.2.2.2	Test parameters	4359

A.14.6.2.2.3	Test Requirements	4360

A.14.6.2.3.2	Test parameters	4360

A.14.6.2.3.3	Test Requirements	4361

A.14.6.2.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode B for Satellite access	4362

A.14.6.2.4.1	Test Purpose and Environment	4362

A.14.6.2.4.2	Test parameters	4362

A.14.6.2.4.3	Test Requirements	4363

Annex B (normative): Conditions for RRM requirements applicability for operating bands	4364

B.1	Conditions for E-UTRAN RRC_IDLE state mobility	4364

B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection	4364

B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection	4364

B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1	4364

B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1	4366

B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1	4367

B.1.6	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category 1bis	4367

B.1.7	Conditions for measurements of E-UTRAN cells for cell re-selection for UE Category M2	4367

B.1.7.1	Conditions for measurements of intra-frequence E-UTRAN cells for cell selection	4367

B.1.7.2	Condition for measurements of inter-frequence E-UTRAN cells for cell selection	4369

B.1.8	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1	4370

B.1.9	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1 for satellite access	4370

B.1.10	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1 and NB2 for satellite access	4371

B.1.11	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1 for satellite access	4372

B.1.12	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1 for satellite access	4372

B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State	4372

B.2.1	Conditions for E-UTRAN intra-frequency measurements	4372

B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps	4373

B.2.3	Conditions for E-UTRAN inter-frequency measurements	4373

B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps	4374

B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements	4374

B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements	4374

B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell	4375

B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction	4375

B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information	4376

B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements	4376

B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements	4376

B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements	4376

B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements	4377

B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	4377

B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	4377

B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3	4378

B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3	4378

B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	4378

B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	4379

B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1	4379

B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1	4380

B.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1	4381

B.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1	4382

B.2.18	Conditions for E-UTRAN inter-frequency measurements by UE Category M1	4383

B.2.19	Conditions for E-UTRAN measurements by UE Category M2	4384

B.2.19.1	Conditions for E-UTRAN intra-frequency measurements	4384

B.2.19.2	Conditions for E-UTRAN inter-frequency measurements	4384

B.2.20	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M1	4384

B.2.21	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M2	4385

B.2.22	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M1	4385

B.2.23	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M2	4387

B.2.24	Conditions for intra-frequency neighbour cell measurements of NB-IoT cells for UE Category NB1	4387

B.2.25	Conditions for inter-frequency neighbour cell measurements of NB-IoT cells for UE Category NB1	4387

B.2.26	Conditions for E-UTRAN intra-frequency measurements by UE Category M1 for satellite access	4387

B.2.27	Conditions for NB-IoT intra-frequency measurements by UE Category NB1 and NB2 for satellite access	4388

B.2.28	Conditions for E-UTRAN inter-frequency measurements by UE Category M1 for satellite access	4389

B.2.29	Conditions for intra-frequency neighbour cell measurements of NB-IoT cells for UE Category NB1 for satellite access	4389

B.2.30	Conditions for inter-frequency neighbour cell measurements of NB-IoT cells for UE Category NB1 for satellite access	4390

B.3	Conditions for measurements performance requirements for UE	4390

B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements	4390

B.3.2	Void	4390

B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements	4391

B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements	4391

B.3.5	Conditions for UE Rx – Tx time difference	4391

B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements	4391

B.3.7	Conditions for inter-frequency RSTD measurements	4391

B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements	4391

B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction	4392

B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction	4392

B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	4392

B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	4392

B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	4392

B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	4392

B.3.14.1	Conditions for Intra-frequency CRS-based measurements	4392

B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements	4393

B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements	4393

B.3.15.1	Conditions for Intra-frequency CRS-based measurements	4393

B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements	4393

B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	4394

B.3.16.1	Conditions for Inter-frequency CRS-based measurements	4394

B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements	4394

B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements	4394

B.3.17.1	Conditions for Inter-frequency CRS-based measurements	4394

B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements	4394

B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements	4395

B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements	4395

B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements	4395

B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	4395

B.3.21.1	Conditions for RSRP measurements	4395

B.3.21.2	Conditions for RSRQ measurements	4395

B.3.21.3	Conditions for CSI-RSRP measurements	4395

B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	4396

B.3.22.1	Conditions for RSRP measurements	4396

B.3.22.2	Void	4396

B.3.22.3	Conditions for CSI-RSRP measurements	4396

B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	4396

B.3.23.1	Conditions for RSRP measurements	4396

B.3.23.2	Conditions for RSRQ measurements	4397

B.3.23.3	Conditions for CSI-RSRP measurements	4397

B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	4397

B.3.24.1	Conditions for RSRP measurements	4397

B.3.24.2	Conditions for RSRQ measurements	4397

B.3.24.3	Conditions for CSI-RSRP measurements	4397

B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	4397

B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	4398

B.3.27	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements for Category 0	4398

B.3.28	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements for Category 0	4398

B.3.29	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for NB1	4399

B.3.30	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for NB1	4399

B.3.31	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	4399

B.3.32	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	4399

B.3.33	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	4399

B.3.34	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	4399

B.4	RRM Requirements Exceptions	4400

B.4.1	General	4400

B.4.2	Receiver sensitivity relaxation for UE supporting CA	4400

B.4.3	Receiver sensitivity relaxation for UE configured with CA	4400

B.4.3.1	Inter-band carrier aggregation	4400

B.4.3.2	Intra-band non-contiguous carrier aggregation	4400

B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band	4400

B.5	Conditions for Measurement Performance Requirements for ProSe UE	4401

B.5.1	Conditions for S-RSRP Accuracy Requirements	4401

B.5.2	Conditions for Relative S-RSRP Accuracy Requirements	4401

B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	4401

B.5.4	Conditions for SD-RSRP Accuracy Requirements	4402

B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements	4402

B.6	Conditions for V2X	4403

B.6.1	Test parameters for GNSS signals	4403

B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements	4403

B.6.3	Conditions for Relative S-RSRP Accuracy Requirements	4403

B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	4403

B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements	4404

B.7	Conditions for sTTI and 1ms-TTI with 3 Subframe HARQ Processing	4404

B.7.1	Conditions for Maximum Timing Difference Between Uplink and Downlink Carriers in Carrier Aggregation	4404

B.8	High level test procedure for SAN RRM tests	4406

Annex C (informative): Change history:	4406