| 3GPP TR 37.829 V0.3.0 (2022-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands for Rel-18 (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	7

3.3	Abbreviations	7

4	Background	7

4.1	General	7

4.2	Justification of the work	7

4.3	Objectives of the work	8

5 	Band specific requirements for a UE	8

5.1	Bands n71 and n85	8

5.1.1	REFSENS exception	9

5.1.2	A-MPR and MPR	9

5.1.2.1	A-MPR Simulation assumptions	9

5.1.2.2	A-MPR Simulation results	10

5.1.2.3	A-MPR	12

5.1.2.4	Edge allocations for PC2 and PC1.5	13

5.1.2.5	MPR Simulation results	16

5.1.2.6	MPR	17

5.1.3	Feasibility of the filter	18

5.1.4	 Feasibility of the PA	18

5.1.4	Other	18

5.2	Band XY	18

Annex A (informative): Change history	19

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is a technical report for release 17 basket WI High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

3GPP	3rd Generation Partnership Project

A-MPR	Additional Maximum Power Reduction

BB	Baseband

CBW	Channel Band Width

CIM	Counter Inter Modulation

FDD	Frequency Division Duplex

HPUE	High Power User Equipment

IMD	Inter Modulation Distortion

LTE	Long Term Evolution

MBW	Measurement Bandwidth

MPR	Maximum Power Reduction

NR	New Radio

NS	Network Signalling

OFDM	Orthogonal Frequency Division Multiplexing

OOB	Out of Band

PA	Power Amplifier

PC1	Power Class 1

RB	Resource Block

SEM	Spectrum Emission Mask

SCS	Sub Carrier Spacing

TDD	Time Division Duplex

TRX	Transceiver

UE	User Equipment

WI	Work Item

# 4	Background

## 4.1	General

Rel-18 WI High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands is a continuation for REL-17 basket WI High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands.

## 4.2	Justification of the work

Support for fixed wireless and vehicle mounted user equipment usage scenarios, with broader rural coverage and higher data rates is envisioned as part of deployment configurations in LTE band 12, band 5 and band 13, and in NR band n5, n26, n13, n71 and n85. Improvements in coverage, availability, and throughput performance to meet the market demands associated with fixed wireless and vehicle mounted usage would be enabled with user equipment specified with a power class 1 (31dBm) up-link transmission capability.

The fixed wireless access scenario provides a variety of benefits consisting of rapid deployment, and a reduction of costs associated with the transport to a customer premise, relative to wireline types of transport. The enablement of fixed wireless user equipment provides backhauling services to an appropriate base station in a serving network, for any other equipment at the customer premise. Similarly, vehicle mounted access scenario provides both direct access to other devices and indirect access to other devices, via the network.

The REL17 WI on High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands is completed with the approval of TR 37.828. REL17 completed all necessary core requirements, most of necessary co-existence studies and concluded release independence aspects.

Therefore, a REL18 WI can be started to further discuss and agree on the corresponding band specific HPUE requirements in the RAN4 specifications.

## 4.3	Objectives of the work

This is a basket WI including following E-UTRA and NR bands.

| Band | contact company (email address) | other supporters (min. 3) | Status | Release independent from |
| --- | --- | --- | --- | --- |
| 5 | sebastian.thalanany@uscellular.com | Nokia, Ericsson and Samsung | Ongoing | Rel-10 |
| 12 | sebastian.thalanany@uscellular.com | Nokia, Ericsson and Samsung | Ongoing | Rel-10 |
| 13 | zheng.zhao@VERIZONWIRELESS.COM | Nokia, Ericsson, Qualcomm and Samsung | Ongoing | Rel-10 |
| n5 | sebastian.thalanany@uscellular.com | Nokia, Ericsson and Samsung | Ongoing | Rel-15 |
| n13 | zheng.zhao@VERIZONWIRELESS.COM | Nokia,,Ericsson, Qualcomm and Samsung | Ongoing | Rel-15 |
| n26 | bill.shvodian@t-mobile.com | Nokia, Ericsson, Deutsche Telekom | Ongoing | Rel-15 |
| n71 | bill.shvodian@t-mobile.com | Nokia, Ericsson, Deutsche Telekom | Ongoing | Rel-15 |
| n85 | bill.shvodian@t-mobile.com | Nokia, Ericsson, Deutsche Telekom | Ongoing | Rel-15 |

Band Specific objectives:

-	UE A-MPR, and impact of other related parameters.

-	see, R4-2210569 WF on A-MPR for bands n71 and n85

-	For other bands which have NS value do A-MPR for protection of other bands study

-	Investigate the feasibility of filter with small duplex for B13 and n13.

The corresponding HPUE requirements for fixed wireless/vehicular-mounted use cases for each can be included in the RAN4 specifications independently when the work on this band is complete, i.e. no need to wait for the completion of other bands.

# 5 	Band specific requirements for a UE

Editor note: This section relates to the Band Specific objectives of the WI.

## 5.1	Bands n71 and n85

Editor note: All sub-clauses are not necessary for all bands.

### 5.1.1	REFSENS exception

### 5.1.2	A-MPR and MPR

#### 5.1.2.1	A-MPR Simulation assumptions

The simulation assumptions are based on the WF [3]:

-	PC1 PA linearity is assumed (37dB ACLR for DFT-s-OFDM QPSK 20MHZ 100RB0 waveform with 1dB MPR).

-	Normal TRX impairments are assumed: 28dB carrier and image leakage, 60dB CIM3 and 70dB CIM5.

-	BB WOLA windowing aspects are covered.

-	At least QPSK DFT-s-OFDM full and edge allocations are evaluated for 5 and 20MHz CBW.

-	Other CBW and allocations are not precluded.

Other assumptions:

-	Channel bandwidths and subcarrier spacings according to TS 38.101-1 [4], Table 5.3.5-1:

Table 5.1.2-1: Additional requirements for "NS_35"

| NR band | SCS [kHz] | UE channel bandwidth [MHz] |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 |
| n71 | 15 | yes | yes | yes | yes |
|  | 30 | - | yes | yes | yes |
| n85 | 15 | yes | yes | yes | - |
|  | 30 | - | yes | yes | - |

-	Both OFDM and DFT-S-OFDM waveforms.

-	All modulations: pi/2-BPSK, QPSK, 16-QAM, 64-QAM, 256-QAM.

-	WOLA processing (window slope is 33 % of the short CP duration).

-	SEM for n71 (NS_35) according to TS 38.101-1 [4], Table 6.5.2.3.1-1:

Table 5.1.2-2: Additional requirements for "NS_35"

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 |  |
| 0-0.1 | -15 | -18 | -20 | -21 | 30 kHz |
| 0.1-6 | -13 | -13 | -13 | -13 | 100 kHz |
| 6-10 | -251 | -13 | -13 | -13 | 100 kHz |
| 10-15 |  | -251 | -13 | -13 | 100 kHz |
| 15-20 |  |  | -251 | -13 | 100 kHz |
| 20-25 |  |  |  | -25 | 1 MHz |
| NOTE 1: The measurement bandwidth shall be 1 MHz |  |  |  |  |  |

-	SEM for n85 (NS_06) according to TS 38.101-1 [4], Table 6.5.2.3.4-1:

Table 5.1.2-3: Additional requirements for "NS_06" or "NS_07"

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 |  |
| ± 0 – 0.1 | -15 | -18 | -20 | 30 kHz |
| ± 0.1 – 1 | -13 | -13 | -13 | 100 kHz |
| ± 1 – 6 | -13 | -13 | -13 | 1 MHz |
| ± 6 – 10 | -25 |  |  |  |
| ± 10 – 15 |  | -25 |  |  |
| ± 15 – 20 |  |  | -25 |  |

-	General requirements as specified in TS 38.101-1 [4].

#### 5.1.2.2	A-MPR Simulation results

All valid channel bandwidth / SCS combinations for n71 and 85, both waveforms, all modulations, all contiguous RB allocations have been simulated. Results of the two bands were quite similar due to the similar SEMs. Examples of the results are shown in Figures 5.1.2-1 to 5.1.2-5.

Figure 5.1.2-1. n71 back-off for 5 MHz, 15 kHz

Figure 5.1.2-2. n71 back-off for 10 MHz, 15 kHz

Figure 5.1.2-3. n71 back-off for 20 MHz, 15 kHz

Figure 5.1.2-4. n85 back-off for 5 MHz, 15 kHz

Figure 5.1.2.2-5. n85 back-off for 15 MHz, 15 kHz

In both n71 and n85, the allocations needing A-MPR are confined to narrow allocations close to either channel edge. Figures 5.1.2-6 and 5.1.2-7 depict two different cases. By coincident, both cases require a similar back-off.

Figure 5.1.2-6. Example spectrum: 10 MHz, OFDM, QPSK, 15 kHz, 1RB@0,
gated by IMD5 hitting the spurious emission limit

Figure 5.1.2-7. Example spectrum: 10 MHz, DFT-S-OFDM, QPSK, 15 kHz, 1RB@0,
gated by regrowth hitting the innermost SEM segment with 30 kHz MBW

#### 5.1.2.3	A-MPR

It is agreed to specify the A-MPR for both n71 and n85 as follows:

-	If RB allocation is within Region A, apply the A-MPR given in Table 5.1.2-4. Otherwise, apply the MPR.

Table 5.1.2-4. PC1 A-MPR for n71 and n85

| Waveform | Modulation | Region A |
| --- | --- | --- |
| DFT-S-OFDM | pi/2-BPSK | 8.5 |
|  | QPSK | 8.5 |
|  | 16-QAM | 8.5 |
|  | 64-QAM | 8.5 |
|  | 256-QAM | 8.5 |
| OFDM | QPSK | 8.5 |
|  | 16-QAM | 8.5 |
|  | 64-QAM | 8.5 |
|  | 256-QAM | 8.5 |

An RB allocation is within Region A if
( LCRB ≤ 0.20 ∙ NRB and ( RBstart = 0 or RBstart + LCRB = NRB ) )
or
( LCRB = 1 and 5 ∙ | RBstart + 0.5 – NRB / 2 | ∙ 12 ∙ SCS ≥ 1.5 ∙ CBW + 5 MHz ).

Above, RBstart and LCRB are the lowest RB index and size of the RB allocation, NRB is the total number of RBs in the channel, and CBW is the channel bandwidth. The colors in the formula are used below when referring to subexpressions. Region A is illustrated in Figure 5.1.2-8.

The first condition covers narrow allocations at channel edge with spectral regrowth hitting the SEM (the vertical / slanted subregions in Figure 8). The second condition covers cases with IMD5 (of the allocation and its image, superpositioned with CIM5) hitting the spurious limit (the horizontal subregions in Figure 5.1.2-8). The expression
| RBstart + 0.5 – NRB / 2 | ∙ 12 ∙ SCS gives the approximate distance of allocation center from channel center (given that LCRB=1). When multiplied with 5, we obtain the distance of the IMD5 component center from the channel center. The spurious region starts at the distance 1.5 ∙ CBW + 5 MHz from the channel center.The allocation ratio limit of 0.20 is required, e.g., in Figure 1 (DFT-S-OFDM) where the total RB count is  NRB = 25 and the greatest allocation size requiring A-MPR is LCRB = 5.

Figure 5.1.2-8. The proposed Region A

#### 5.1.2.4	Edge allocations for PC2 and PC1.5

As previously discussed in WF [3], if PC1 PA are intrinsically more linear than PC2 and PC3 PAs with an ACLR requirement 37dB of versus 31dB and 30dB respectively, FWA and potentially vehicle mounted UEs may still reuse the BB IC and the RF transceiver from a smartphone platform. This means that both TRX carrier, image and CIM3/5 impairments apply but also the BB WOLA filtering will seek the same compromise that is found in smartphone platforms in terms of transient performance and margin to the channel guard-band on the waveform spectrum.

If there are agreed assumptions for carrier, image and CIM3/5 impairments, there is no agreed WOLA filtering. Still, some information is available through the PC2 and PC3 experience.

Edge allocations were first introduced for PC2 MPR for QPSK and higher order modulations. Only later this concept was reused for PC3 in the context of Pi/2 BPSK with power boosting.

If the different MPR tables for PC3, PC2, PC1.5 and PC1 extracted from 38.101-1 [4] and copied at the bottom are compared, and focus for QPSK modulation, it can be found that:

-	PC3 does not have edge allocations and outer MPR is 1dB and 3dB for DFT and CP respectively.

-	PC2 has edge allocation MPR of minimum 3.5dB across all modulations and outer MPR is 1dB and 3dB for DFT and CP respectively. Edge allocations are allocations within the two edge RBs on each side of the channel.

-	For PC1.5 (using FWA case to neglect issues related to reverse IMD) has edge allocation MPR of minimum 6.5dB across all modulations and outer MPR is 2dB and 4dB for DFT and CP respectively. Edge allocations are allocations within the four edge RBs on each side of the channel.

-	PC1 MPR does not have edge allocations but if the same transceiver and BB would be assumed than for PC2 and PC1.5, then it would be needed.

It can be observed that while outer MPR is relatively constant across PC3 to PC1.5, edge A-MPR is increased dB/dB with the power class level, it is constant across modulation orders and edge RB region doubles in size every 3dB of PCmax. This means that this is not related to the transmit path linearity, but the consequence of the input spectrum from the BB.

With the above observation and if the same BB WOLA filtering would be assumed, the PC1 MPR should also have an edge allocation type at 8.5dB minimum MPR (2dB above PC1.5) and cover the 7RB at both edge of the channel (4*10^(2/10) is 6.4 and corresponds to a 2dB increase of the edge region).

Note that this edge allocation MPR is worst for channel bandwidths with lower guard-bands, but when considering A-MPR, one should further check these allocations as some NS have more stringent requirements in the OOB first 1MHz, either in terms of level or measurement bandwidth.

Therefore, it is agreed on edge allocations:

-	If PC1 MPR needs to be representative of implementations using BB and RF transceivers from smartphone platforms, it should consider the addition of an edge allocation type with a minimum MPR of 8.5dB and an edge region of seven RBs.

-	It may be feasible to agree a slightly better value of 8dB or add signaling for the UE to declare the need for this additional allocation type.

-	Edge allocations with similar WOLA performance than evaluated for PC2 should be carefully checked for NS where the requirement in the first OOB MHz is more stringent or use smaller measurement bandwidth than NR SEM.; especially for channel bandwidths with small guard-bands.

Table 6.2.2-1 Maximum power reduction (MPR) for power class 3

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.51 | ≤ 1.21 | ≤ 0.21 |
|  |  | ≤ 0.52 | ≤ 0.52 | 02 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.52 | 02 | 02 |
|  | QPSK | ≤ 1 |  | 0 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: Applicable for UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and if the IE powerBoostPi2BPSK is set to 1 and 40 % or less slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. The reference power of 0 dB MPR is 26 dBm. NOTE 2: Applicable for UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79 with Pi/2 BPSK modulation and if the IE powerBoostPi2BPSK is set to 0 and if more than 40 % of slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. |  |  |  |  |

Table 6.2.2-2 Maximum power reduction (MPR) for power class 2

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 0.5 | 0 |
|  | QPSK | ≤ 3.5 | ≤ 1 | 0 |
|  | 16 QAM | ≤ 3.5 | ≤ 2 | ≤ 1 |
|  | 64 QAM | ≤ 3.5 | ≤ 2.5 |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3 | ≤ 1.5 |
|  | 16 QAM | ≤ 3.5 | ≤ 3 | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

Table 6.2D.2-3 Maximum power reduction (MPR) for power class 1.5 with dual Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6 | ≤ 1.5 | ≤ 0 |
|  | QPSK | ≤ 6.5 | ≤ 2 | ≤ 0 |
|  | 16 QAM | ≤ 6.5 | ≤ 3 | ≤ 1 |
|  | 64 QAM | ≤ 6.5 | ≤ 3.5 | ≤ 3 |
|  | 256 QAM | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 4 | ≤ 1.5 |
|  | 16 QAM | ≤ 6.5 | ≤ 4 | ≤ 2 |
|  | 64 QAM | ≤ 6.5 | ≤ 4.5 | ≤ 4 |
|  | 256 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |
| NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |  |  |  |

Table 6.2.2-5 Maximum power reduction (MPR) for power class 1

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 0.5 | ≤ 0.5 | 0 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.5 | 0 | 0 |
|  | QPSK | ≤ 1 |  | 0 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

#### 5.1.2.5	MPR Simulation results

Figure 5.1.2-9 shows the simulated power back-off for SEM-limited RB allocations at or close to channel edge, due to linear spectral regrowth caused by the windowing effect. Each marker represents a combination of CBW / waveform / modulation; its value is the maximum back-off among all combinations of SCS / RB allocation. Included are only parameter combinations that exceed the respective MPR of the given RB allocation.

Figure 5.1.2-9 shows a clear dependence between the back-off and CBW.

Figure 5.1.2-9: Simulated back-off and allowed MPR

For the SEM-limited RB allocations, the back-off is determined primarily by two factors:

-	The emission limit in the first SEM segment at channel edge: The SEM limit is -13 dBm below 50 MHz CBW and -24 dBm from 50 MHz upwards (from Table 6.5.2.2-1 in TS 38.101-1 [4]). This can be seen as a sudden rise of back-off at 50 MHz CBW.

-	The guard-to-SCS ratio, i.e, the ratio of minimum guard band width (from TS 38.101-1, Table 5.3.3-1) to the SCS: On the frequency axis, the linear spectral regrowth due to windowing scales with the SCS. The smaller the guard-to-SCS ratio, the higher emission and thus the higher required power back-off. Below 50 MHz CBW, the guard-to-SCS ratio mostly increases with increasing CBW, resulting in decreasing needed back-off. From 50 MHz upwards, the guard-to-SCS ratio mostly decreases with increasing CBW, resulting in increasing needed back-off.

From the simulation data, the allowed MPR can be defined as described in clause 5.1.2.6. This will avoid allowing MPR that is more than necessary. For example, adopting a proposal to define the allowed MPR with a common value for all CBW <50 MHz and a common value for all CBW ≥50 MHz will result in a 2dB power back-off more than necessary for 45 MHz and 50 MHz CBW which, in turn, would lead to 2dB UL coverage loss. This is illustrated as green lines in Figure 5.1.2-9.

The Edge RB allocations are narrow allocations at or close to channel edge, so that their linear spectral regrowth reaches beyond the channel edge and its PSD is sufficient to violate the SEM. The required maximum RB allocation size of Edge RB allcoations is illustrated in Figure 5.1.2-10.

Figure 5.1.2-10: Maximum allocation size of Edge RB allocations as function of channel bandwidth

From the simulation data, the set of Edge RB allocations can be specified separately for all CBW <50 MHz and for all CBW ≥50 MHz as described in clause 5.1.2.6.

#### 5.1.2.6	MPR

It is agreed to specify the FR1 PC1 MPR for all operating bands as follows, except for band n14 MPR which is keep as it is:

-	Specify the MPR for Edge RB allocations as follows:

-	For PC1 UE supporting other bands than n14 the MPR for Edge RB allocation is defined as follows for two distinguished channel bandwidths groups as:

Within the <50MHz channel bandwidth group:

MPR= CEIL(7.2 dB- 6 dB∙ CBW100 MHz, 0.5 dB)

Within the ≥50MHz channel bandwidth group:

MPR= CEIL5.35 dB+3.15 dB∙ CBW100 MHz, 0.5 dB

where CEIL(x,0.5 dB) means rounding x upwards to the closest multiple of 0.5 dB.

-	Specify the set of Edge RB allocations as follows:

-	For PC1 UE supporting other bands than n14 RB allocation is an Edge RB allocation if

LCRB≤LCRB,edge AND ( RBstart≤RBstart,edge OR RBstart≥NRB-RBstart,edge-LCRB ),

where

LCRB,edge=6, CBW<50 MHz12, CBW≥50 MHz.

For CBW≥70 MHz with DFT-S-OFDM waveform and pi/2-BPSK, QPSK, or 16-QAM modulation, RBstart,edge=1. Otherwise, RBstart,edge=0.

### 5.1.3	Feasibility of the filter

### 5.1.4		Feasibility of the PA

### 5.1.4	Other

## 5.2	Band XY

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-08 | RAN4#104-e | R4-2213758 |  |  |  | Skeleton TR | 0.0.1 |
| 2022-08 | RAN4#104-e | R4-2213759 |  |  |  | TP to TR 37.389: Background information. | 0.1.0 |
| 2022-08 | RAN4#104-e | R4-2214432 |  |  |  | TP to TR 37.829: PC1 A-MPR and MPR for bands n71 and n85 | 0.1.0 |
| 2022-10 | RAN4#104-bis-e | R4-2215319 |  |  |  | TP for TR 37.389 to add abbreviations | 0.2.0 |
| 2022-11 | RAN4#105 | R4-2218323 |  |  |  | TP to TR 37.829: PC1 A-MPR and MPR for bands n71 and n85 | 0.3.0 |