| 3GPP TR 37.898 V0.3.0 (2025-02) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; High power UE (power class 1.5 or 2) for Dual Connectivity (DC) combinations of LTE band(s) and NR band(s) ; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Background	9

4.1	TR Maintenance	9

5	High power UE (power class 2) for EN-DC	9

5.1	DC_3-11_n79	9

5.1.1	Configuration for DC	9

5.1.2	Maximum output power for DC	9

5.1.3	REFSENS requirements for DC	9

5.1.4	∆TIB and ∆RIB values	10

5.2	DC_8-42_n79	10

5.2.1	Configuration for DC	10

5.2.2	Maximum output power for DC	10

5.2.3	REFSENS requirements for DC	10

5.2.4	∆TIB and ∆RIB values	11

5.3	DC_1A_n78A	11

5.3.1	Configurations for DC	11

5.3.2	Maximum output power for DC	11

5.3.3	Maximum output power for DC with UL MIMO	11

5.3.4	Maximum output power for DC with Tx Diversity	12

5.3.5	REFSENS requirements for DC	12

5.3.6	∆TIB and ∆RIB values	12

5.4	DC_5A_n78A	12

5.4.1	Configurations for DC	12

5.4.2	Maximum output power for DC	13

5.4.3	Maximum output power for DC with UL MIMO	13

5.4.4	Maximum output power for DC with Tx Diversity	13

5.4.5	REFSENS requirements for DC	13

5.4.6	∆TIB and ∆RIB values	14

5.5	DC_(n)40	14

5.5.1	Configurations	14

5.5.2	Maximum output power	14

5.5.3	REFSENS requirements for DC	14

5.5.4	∆TIB and ∆RIB values	14

5.6	DC_4A_n41A	15

5.6.1	Configurations	15

5.6.2	Maximum output power	15

5.6.3	REFSENS requirements	15

5.6.4	∆TIB and ∆RIB values	16

5.7	DC_4A_n78A	16

5.7.1	Configurations	16

5.7.2	Maximum output power	16

5.7.3	REFSENS requirements	16

5.7.4	∆TIB and ∆RIB values	16

5.8	DC_7C_n78A	17

5.8.1	Configurations	17

5.8.2	Maximum output power	17

5.8.3	REFSENS requirements	17

5.8.4	∆TIB and ∆RIB values	17

5.9	DC_8A_n41A	18

5.9.1	Configurations	18

5.9.2	Maximum output power	18

5.9.3	REFSENS requirements	18

5.9.4	∆TIB and ∆RIB values	19

5.10	DC_20A_n41A	19

5.10.1	Configurations	19

5.10.2	Maximum output power	19

5.10.3	REFSENS requirements	20

5.10.4	∆TIB and ∆RIB values	20

5.11	DC_20A_n78A	20

5.11.1	Configurations	20

5.11.2	Maximum output power	21

5.11.3	REFSENS requirements	21

5.11.4	∆TIB and ∆RIB values	21

5.12	DC_38A_n78A	21

5.12.1	Configurations	21

5.12.2	Maximum output power	22

5.12.3	REFSENS requirements	22

5.12.4	∆TIB and ∆RIB values	22

5.13	DC_41_n78	23

5.13.1	Configurations	23

5.13.2	Maximum output power	23

5.13.3	REFSENS requirements	23

5.13.4	∆TIB and ∆RIB values	23

5.14	DC_11A_n41A	24

5.14.1	Configurations	24

5.14.2	Maximum output power	24

5.14.3	REFSENS requirements	24

5.14.4	∆TIB and ∆RIB values	24

5.15	DC_12A_n41A	25

5.15.1	Configurations	25

5.15.2	Maximum output power	25

5.15.3	REFSENS requirements	25

5.15.4	∆TIB and ∆RIB values	25

5.16	DC_12A_n78A	26

5.16.1	Configurations	26

5.16.2	Maximum output power	26

5.16.3	REFSENS requirements	26

5.16.4	∆TIB and ∆RIB values	27

5.17	DC_18A_n78A	27

5.17.1	Configurations	27

5.17.2	Maximum output power	27

5.17.3	REFSENS requirements	27

5.17.4	∆TIB and ∆RIB values	28

5.18	DC_25A_n41A	28

5.18.1	Configurations	28

5.18.2	Maximum output power	28

5.18.3	REFSENS requirements	28

5.18.4	∆TIB and ∆RIB values	29

5.19	DC_25A_n78A	29

5.19.1	Configurations	29

5.19.2	Maximum output power	29

5.19.3	REFSENS requirements	29

5.19.4	∆TIB and ∆RIB values	30

5.20	DC_26A_n78A	30

5.20.1	Configurations	30

5.20.2	Maximum output power	31

5.20.3	REFSENS requirements	31

5.20.4	∆TIB and ∆RIB values	31

5.21	DC_40A_n41A	32

5.21.1	Configurations	32

5.21.2	Maximum output power	32

5.21.3	REFSENS requirements	32

5.21.4	∆TIB and ∆RIB values	32

6	High power UE (power class 1.5) for EN-DC	33

Annex <A> (informative): Change history	34

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is one of four technical reports for Rel-19 High power UE (power class 1.5 or 2) for Dual Connectivity (DC) combinations of LTE band(s) and NR band(s) under Rel-19 time frame. The purpose is to gather the relevant background information and studies to address relevant requirements for the Rel-19 EN-DC HPUE band combinations requested by proponents and captured in the WID.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP RP-241678: “Rel-19 High power UE (power class 1.5 or 2) for Dual Connectivity (DC) combinations of LTE band(s) and NR band(s)”.

[3]	R4-2318721 “TP for TR 38.880: DC_20A_n78A with 3Tx” Huawei, HiSilicon

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c.

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA operation, for serving cell c.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

BS				Base Station

BCS				Bandwidth Combination Set

CA				Carrier Aggregation

CC				Component Carriers

DC				Dual Connectivity

DL				Downlink

FDD				Frequency Division Duplex

MPR 				Allowed maximum power reduction

MSD				Maximum Sensitivity Degradation

REFSENS				Reference Sensitivity power level

SCS				Subcarrier spacing

SUL 				Supplementary uplink

TDD				Time Division Duplex

UE				User Equipment

UL				Uplink

# 4	Background

At 3GPP RAN#104 meeting, a basket Work Item on “Rel-19 High power UE (power class 1.5 or 2) for Dual Connectivity (DC) combinations of LTE band(s) and NR band(s)” [2] was approved for Rel-19. The present document is a technical report for this basket Work Item.

## 4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band/band combination to ensure that the TPs related to the band/band combination have been implemented.

# 5	High power UE (power class 2) for EN-DC

## 5.1	DC_3-11_n79

### 5.1.1	Configuration for DC

Table 5.1.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-11A_n79A14 | DC_3A_n79A14 |
| NOTE1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

### 5.1.2	Maximum output power for DC

The maximum output power requirement for PC2 UL DC_3_n79 is already specified in Table 6.2B.1.3-1 in TS38.101-3[3]. So, this section can be omitted.

### 5.1.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_3_n79 and DC_11_n79 captured in TR37.863-01-01[5], own Rx impact of the 3 bands is as follows:

-	3rd order IMD generated by dual uplink of band 3 and band n79 may fall into own Rx of band 3 and Rx Band of 11.

Table 5.1.3-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-11A_n79A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | 11 | N/A | 5 | 25 | N/A | 25.1 | IMD3 |
|  | n79 | 4920 | 40 | 216 | 4920 | N/A | N/A |

### 5.1.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC.

## 5.2	DC_8-42_n79

### 5.2.1	Configuration for DC

Table 5.2.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_8A-42A_n79A14 | DC_8A_n79A14 |
| NOTE1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

### 5.2.2	Maximum output power for DC

The maximum output power requirement for PC2 UL DC_8_n79 is already specified in Table 6.2B.1.3-1 of TS38.101-3[3]. So, this section can be omitted.

### 5.2.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_8_n79 and DC_42_n79 captured in TR37.863-01-01[5], own Rx impact of the 3 bands is as follows:

-	2nd order IMD generated by dual uplink of band 8 and band n79 may fall into own Rx of band 42.

-	3rd order IMD generated by dual uplink of band 8 and band n79 may also fall into own Rx of band 79. However, TDD band does not transmit and receive simultaneously in a single band, thus the own band of n79 would not be interfered.

-	3rd order IMD generated by dual uplink of band 42 and band n79 may also fall into own Rx of band 42. However, UL of band 42 is not used in this configuration. So, there is no own Rx impact of 3rd band for this configuration.

Table 5.2.3-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_8A-42A_n79A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n79 | 4470 | 40 | 216 | 4470 | N/A | N/A |
|  | 42 | N/A | 5 | N/A | 3570 | 34.8 | IMD2 |

### 5.2.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC.

## 5.3	DC_1A_n78A

### 5.3.1	Configurations for DC

Table 5.3.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A_n78A7, 23 | DC_1A_n78A21, 23 |
| NOTE 7: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. NOTE 21: Minimum requirements for PC2 are applicable for this uplink EN-DC configuration in this downlink/uplink EN-DC configuration with 1Tx antenna connector in each band. NOTE 23: Minimum requirements for Power Class 2 are applicable for this EN-DC configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. |  |

We herein assume that Table 5.3.1-1 of this section corresponds to Table 5.5B.4.1-1 of TS 38.101-3 V18.6.0. Then, to specify 3Tx, we add NOTE 23 to this configuration while NOTE 7 and NOTE 21 are originally there.

### 5.3.2	Maximum output power for DC

Table 5.3.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 1.5 (dBm) | Tolerance (dB) | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |  |  |  |  |

Please note that the maximum output power for DC_1A_n78A was already specified in Table 6.2B.1.3-1 of TS 38.101-3 V18.6.0. To specify 3Tx for this configuration, we do not need to add any new information to Table 5.3.2-1 of this section.

### 5.3.3	Maximum output power for DC with UL MIMO

Table 5.3.3-1: Maximum output power for inter-band EN-DC with UL MIMO (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_1A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| NOTE 5: The UE supports PC3 in E-UTRA band, and supports PC3 or PC2 with UL MIMO in NR band. NOTE 7: FWA form factor is targeted unless otherwise stated. |  |  |  |  |

To specify 3Tx for DC_1A_n78A, we add this configuration with NOTE 5 to Table 5.3.3-1 of this section, which corresponds to Table 6.2H.1.3-1 of TS 38.101-3 V18.6.0.

### 5.3.4	Maximum output power for DC with Tx Diversity

Table 5.3.4-1: Maximum output power for inter-band EN-DC with Tx Diversity (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_1A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| NOTE 5: The UE supports PC3 in E-UTRA band, and supports PC3 or PC2 with Tx Diversity in NR band. NOTE 7: FWA form factor is targeted unless otherwise stated. |  |  |  |  |

To specify 3Tx for DC_1A_n78A, we add this configuration with NOTE 5 to Table 5.3.4-1 of this section, which corresponds to Table 6.2L.1.3-1 of TS 38.101-3 V18.6.0.

### 5.3.5	REFSENS requirements for DC

Based on the UE coexistence study of Band 1 and Band n78, we identify the following interference impact:

-	The IMD4 caused by the 3rd order of Band 1 UL and the 1st order of Band n78 UL may impact on Band 1 DL.

-	For this interference, the existing MSD values of PC3 and PC2 DC_1A_n78A, which were already specified in Table 7.3B.2.3.5.1-1 and Table 7.3B.2.3.5.1-1a of TS 38.101-3 V18.6.0, respectively, can be applied to DC_1A_n78A with 3Tx.

### 5.3.6	∆TIB and ∆RIB values

The existing ∆TIB and ∆RIB values of DC_1_n78, which were already specified in Table 6.2B.4.2.3.1-1 and Table 7.3B.3.3.1-1 of TS 38.101-3 V18.6.0, respectively, can be applied to DC_1A-n78A with 3Tx.

## 5.4	DC_5A_n78A

### 5.4.1	Configurations for DC

Table 5.4.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_5A_n78A7, 23 | DC_5A_n78A23 |
| NOTE 7: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. NOTE 23: Minimum requirements for Power Class 2 are applicable for this EN-DC configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. |  |

We herein assume that Table 5.4.1-1 of this section corresponds to Table 5.5B.4.1-1 of TS 38.101-3 V18.6.0. Then, to specify 3Tx, we add NOTE 23 to this configuration while NOTE 7 is originally there.

### 5.4.2	Maximum output power for DC

Table 5.4.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 1.5 (dBm) | Tolerance (dB) | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_5A_n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |  |  |  |  |

Please note that the maximum output power for DC_5A_n78A was already specified in Table 6.2B.1.3-1 of TS 38.101-3 V18.6.0. To specify 3Tx for this configuration, we do not need to add any new information to Table 5.4.2-1 of this section.

### 5.4.3	Maximum output power for DC with UL MIMO

Table 5.4.3-1: Maximum output power for inter-band EN-DC with UL MIMO (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_5A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| NOTE 5: The UE supports PC3 in E-UTRA band, and supports PC3 or PC2 with UL MIMO in NR band. NOTE 7: FWA form factor is targeted unless otherwise stated. |  |  |  |  |

To specify 3Tx for DC_5A_n78A, we add this configuration with NOTE 5 to Table 5.4.3-1 of this section, which corresponds to Table 6.2H.1.3-1 of TS 38.101-3 V18.6.0.

### 5.4.4	Maximum output power for DC with Tx Diversity

Table 5.4.4-1: Maximum output power for inter-band EN-DC with Tx Diversity (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_5A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| NOTE 5: The UE supports PC3 in E-UTRA band, and supports PC3 or PC2 with Tx Diversity in NR band. NOTE 7: FWA form factor is targeted unless otherwise stated. |  |  |  |  |

To specify 3Tx for DC_5A_n78A, we add this configuration with NOTE 5 to Table 5.4.4-1 of this section, which corresponds to Table 6.2L.1.3-1 of TS 38.101-3 V18.6.0.

### 5.4.5	REFSENS requirements for DC

Based on the UE coexistence study of Band 5 and Band n78, we identify the following interference impact:

-	The 4th order harmonic of Band 5 UL may have an impact on the 1st order harmonic of Band n78 DL. It corresponds to UL harmonic interference.

-	For this interference, the existing MSD value of DC_5A_n78A, which was already specified in Table 7.3B.2.3.1-1 of TS 38.101-3 V18.6.0, can be applied to DC_5A_n78A with 3Tx.

-	The 1st order harmonic of Band n78 UL may have an impact on the 4th order harmonic of Band 5 DL. It corresponds to receiver harmonic mixing.

-	In case where the aggressor is PC2, the existing MSD value of DC_5A_n78A, which was already specified in Table 7.3B.2.3.2-1a of TS 38.101-3 V18.6.0, can be applied to DC_5A_n78A with 3Tx.

-	The IMD4 caused by the 3rd order of Band 5 UL and the 1st order of Band n78 UL may impact on Band 5 DL.

-	For this interference, the existing MSD values of PC3 and PC2 DC_5A_n78A, which were already specified in Table 7.3B.2.3.5.1-1 and Table 7.3B.2.3.5.1-1a of TS 38.101-3 V18.6.0, respectively, can be applied to DC_5A_n78A with 3Tx.

### 5.4.6	∆TIB and ∆RIB values

The existing ∆TIB and ∆RIB values of DC_5_n78, which were already specified in Table 6.2B.4.2.3.1-1 and Table 7.3B.3.3.1-1 of TS 38.101-3 V18.6.0, respectively, can be applied to DC_5A-n78A with 3Tx.

## 5.5	DC_(n)40

### 5.5.1	Configurations

Table 5.5.1-1: Intra-band contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_(n)40AA7 | DC_(n)40AA6,7 | Yes6 |
| NOTE 6: Only single switched UL is supported NOTE 7: The PC2 Uplink EN-DC configuration supported in Table 6.2B.1.1-1 is applicable to the same EN-DC configuration. |  |  |

### 5.5.2	Maximum output power

Table 5.5.2-1: Maximum output power for EN-DC (continuous sub-blocks)

| EN-DC configuration | Power class 1.5 (dBm) | Tolerance (dB) | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_(n)40AA3 |  |  | 26 | +2/-3 | 23 | +2/-3 |
| NOTE 3: Only single switched UL is supported. |  |  |  |  |  |  |

### 5.5.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is not needed due to higher power UL DC. Since DC_(n)40AA is to support only single switched UL, there should be no MSD interference issues occurred. Hence, additional MSD is not needed.

### 5.5.4	∆TIB and ∆RIB values

ΔTIB,c and ΔRIB,c are not applicable for intra-band contiguous EN-DC, so this section is omitted.

## 5.6	DC_4A_n41A

### 5.6.1	Configurations

Table 5.6.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed | DL interruption allowed (Note 14) |
| --- | --- | --- | --- |
| DC_4A_n41A | DC_4A_n41A | No |  |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. |  |  |  |

### 5.6.2	Maximum output power

Table 5.6.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_4A_n41A | 266 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |  |  |

### 5.6.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Interference impact is identified as follows:

-	IMD4 may fall into Rx of band n41 for DC_4A_n41A, but there is no impact because of TDD band.

-	There is no IMD and harmonic issue for DC_4A_n41A.

The MSD due to the cross band isolation can be taken from a similar band combination such DC_66_n41A for PC2, as given below.

Table 5.6.3.-1: Reference sensitivity exceptions (MSD) due to cross band isolation and uplink/downlink configurations for PC2 EN-DC in NR FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n41 | 4 | 2546 | 100 | 30 | 270 (RBstart=0) | 2152.5 | 5 | 12.4 | >ACLR2 |

### 5.6.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 5.7	DC_4A_n78A

### 5.7.1	Configurations

Table 5.7.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed | DL interruption allowed (Note 14) |
| --- | --- | --- | --- |
| DC_4A_n78A | DC_4A_n78A | No |  |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. |  |  |  |

### 5.7.2	Maximum output power

Table 5.7.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_4A_n78A | 266 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |  |  |

### 5.7.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Interference impact is identified as follows:

IMD4/IMD5 may fall into Rx of band n78 for DC_4A_n78, but there is no impact because of TDD band.

2nd harmonic interference may fall into Rx of band n78 when band 4 transmit, since only PC3 is specified on Band 4, new MSD due to harmonic interference is not needed.

### 5.7.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 5.8	DC_7C_n78A

### 5.8.1	Configurations

Table 5.8.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed | DL interruption allowed (Note 14) |
| --- | --- | --- | --- |
| DC_7A_n78A7,23 DC_7C_n78A7,21 DC_7A_n78C7 | DC_7A_n78A21,23 DC_7C_n78A | No |  |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 7: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. NOTE 21: Minimum requirements for PC2 are applicable for this uplink EN-DC configuration in this downlink/uplink EN-DC configuration with 1Tx antenna connector in each band. NOTE 23: Minimum requirements for Power Class 2 are applicable for this EN-DC configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. |  |  |  |

### 5.8.2	Maximum output power

Table 5.8.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_7C_n78A | 266 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |  |  |

### 5.8.3	REFSENS requirements

The MSD is already available in Table 7.3B.2.3.4-1a of TS 38.101-3.

### 5.8.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 5.9	DC_8A_n41A

### 5.9.1	Configurations

Table 5.9.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed | DL interruption allowed (Note 14) |
| --- | --- | --- | --- |
| DC_8A_n41A | DC_8A_n41A | No | No |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 14: Applicable when dynamic switching between two uplink carriers is conducted. The DL interruption requirements for NR DL carrier(s) and E-UTRA DL carrier(s) are specified in clause 8.2.1.2.14 of 38.133 [15] and clause 7.32.2.12 of 36.133 [16] respectively. |  |  |  |

### 5.9.2	Maximum output power

Table 5.9.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_8A_n41A | 266 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |  |  |

### 5.9.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Interference impact is identified as follows:

-	the 3rd order harmonic of Band 8 will fall into Band 41.

-	the 3rd and 5th order IMD may fall into Rx frequencies of band 8 and the 5th order IMD may fall into Rx frequencies of Band 41. However IMD is not an issue for Band 41 since it is a TDD band.

-	MSD need to be considered only for Band 8. As only PC3 for band 8 is specified, there is no new MSD due to harmonic.

New MSDs are defined in the following tables.

Table 5.9.3-1: MSD test point for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_8A_n41A | 8 | 882.5 | 5 | 25 | 927.5 | 18.2 | IMD31 |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

### 5.9.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 5.10	DC_20A_n41A

### 5.10.1	Configurations

Table 5.10.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed | DL interruption allowed (Note 14) |
| --- | --- | --- | --- |
| DC_20A_n41A | DC_20A_n41A | DC_20_n41 |  |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 14: Applicable when dynamic switching between two uplink carriers is conducted. The DL interruption requirements for NR DL carrier(s) and E-UTRA DL carrier(s) are specified in clause 8.2.1.2.14 of 38.133 [15] and clause 7.32.2.12 of 36.133 [16] respectively. |  |  |  |

### 5.10.2	Maximum output power

Table 5.10.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_20A_n41A | 266 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |  |  |

### 5.10.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Interference impact is identified as follows:

-	3rd IMD products may also fall into Rx frequencies of band 20.

-	5th IMD products may also fall into Rx frequencies of band 20.

New MSDs are defined in the following tables.

Table 5.10.3-1: MSD test point for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_20A_n41 | 20 | 851 | 5 | 25 | 810 | 19.1dB | IMD 3 |
|  | n41 | 2512 | 10 | 50 | 2512 | N/A | N/A |
| DC_20A_n41 | 20 | 841 | 5 | 25 | 800 | 20.3dB | IMD 5 |
|  | n41 | 2564 | 10 | 50 | 2564 | N/A | N/A |

### 5.10.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 5.11	DC_20A_n78A

### 5.11.1	Configurations

Table 5.11.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed | DL interruption allowed (Note 14) |
| --- | --- | --- | --- |
| DC_20A_n78A | DC_20A_n78A | No |  |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. |  |  |  |

### 5.11.2	Maximum output power

Table 5.11.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_20A_n78A | 266 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |  |  |

### 5.11.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. MSD analysis is done in DC_20A_n78A with 3Tx[3] and reused here:

Table 5.11.3-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_20_n78 | 20 | 850 | 5 | 25 | 809 | 18.8 | IMD4 |
|  | n78 | 3359 | 10 | 50 | 3359 | N/A | N/A |

### 5.11.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 5.12	DC_38A_n78A

### 5.12.1	Configurations

Table 5.12.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed | DL interruption allowed (Note 14) |
| --- | --- | --- | --- |
| DC_38A_n78A | DC_38A_n78A | No |  |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. |  |  |  |

### 5.12.2	Maximum output power

Table 5.12.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_38A_n78A | 266 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |  |  |

### 5.12.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Interference impact is identified as follows:

-	There is no IMD and harmonic issue for DC_38A_n78A.

The MSD due to the cross band isolation can be specified as below:

Table 5.12.3.-1: Reference sensitivity exceptions (MSD) due to cross band isolation and uplink/downlink configurations for PC2 EN-DC in NR FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n78 | 38 | 3350 | 100 | 30 | 270 (RBSTART=0) | 2617.5 | 5 | 5.3 | >ACLR2 |

### 5.12.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 5.13	DC_41_n78

### 5.13.1	Configurations

Table 5.13.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed | DL interruption allowed (Note 14) |
| --- | --- | --- | --- |
| DC_41A_n78A DC_41C_n78A21 | DC_41A_n78A21 DC_41C_n78A | No |  |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 21: Minimum requirements for PC2 are applicable for this uplink EN-DC configuration in this downlink/uplink EN-DC configuration with 1Tx antenna connector in each band. NOTE 22: The PC2 Uplink EN-DC configuration supported in Table 6.2B.1.3-1 is applicable to the same EN-DC configuration without additional indication of NOTE 21. |  |  |  |

### 5.13.2	Maximum output power

Table 5.13.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_41A_n78A | 266 | +2/-3 | 23 | +2/-3 |
| DC_41C_n78A | 266 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |  |  |

### 5.13.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC.

The MSD due to harmonic mixing and the cross band isolation should be considered, as MSD of the band combination has been introduced by 3Tx, the detail MSD requirements for PC2 EN-DC are already available in Table 7.3B.2.3.2-1a and Table 7.3B.2.3.4-1a of TS 38.101-3.

### 5.13.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 5.14	DC_11A_n41A

### 5.14.1	Configurations

Table 5.14.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed | DL interruption allowed (Note 14) |
| --- | --- | --- | --- |
| DC_11A_n41A | DC_11A_n41A | No |  |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. |  |  |  |

### 5.14.2	Maximum output power

Table 5.14.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_11A_n41A | 266 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |  |  |

### 5.14.3	REFSENS requirements

There is no REFSENS exceptions or MSD requirements due to higher power UL DC.

### 5.14.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 5.15	DC_12A_n41A

### 5.15.1	Configurations

Table 5.15.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed | DL interruption allowed (Note 14) |
| --- | --- | --- | --- |
| DC_12A_n41A | DC_12A_n41A | No |  |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. |  |  |  |

### 5.15.2	Maximum output power

Table 5.15.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_12A_n41A | 266 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |  |  |

### 5.15.3	REFSENS requirements

There is no REFSENS exceptions or MSD requirements due to higher power UL DC.

### 5.15.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 5.16	DC_12A_n78A

### 5.16.1	Configurations

Table 5.16.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed | DL interruption allowed (Note 14) |
| --- | --- | --- | --- |
| DC_12A_n78A | DC_12A_n78A | No |  |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. |  |  |  |

### 5.16.2	Maximum output power

Table 5.16.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_12A_n78A | 266 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |  |  |

### 5.16.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Since MSD of CA_n12-n77 is already specified, it can be reused here.

Table 5.16.3-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for PC2 EN-DC in NR FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | 12 | 10 | 15 | 25 | 5 | 34 | NOTE 2 | UL1/DL5 |
| n78 | 12 | 10 | 15 | 25 | 15 | 31 | NOTE 2 | UL1/DL5 |
| NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that and with the DL carrier frequency in the lower band and the UL carrier frequency and the channel bandwidth configured in the higher band, both in MHz. |  |  |  |  |  |  |  |  |

Table 5.16.3-2: MSD test point for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A_n78A | n12 | 702 | 5 | 20 | 732 | 11.7 | IMD5 |
|  | n78 | 3540 | 10 | 50 | 3540 | N/A |  |

### 5.16.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 5.17	DC_18A_n78A

### 5.17.1	Configurations

Table 5.17.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed | DL interruption allowed (Note 14) |
| --- | --- | --- | --- |
| DC_18A_n78A | DC_18A_n78A | No |  |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. |  |  |  |

### 5.17.2	Maximum output power

Table 5.17.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_18A_n78A | 266 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |  |  |

### 5.17.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Since MSD of CA_n18-n77 is already specified, it can be reused here.

Table 5.17.3-1: MSD test point for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_18A_n78A | 18 | 827.5 | 5 | 25 | 872.5 | 18.4 | IMD411 |
|  | n78 | 3355 | 10 | 50 | 3355 | N/A | N/A |
| NOTE 11: For a UE which supports this band combination only when the Band n78 frequency range restriction of 3400 - 3800 MHz in Japan applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |

### 5.17.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 5.18	DC_25A_n41A

### 5.18.1	Configurations

Table 5.18.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed | DL interruption allowed (Note 14) |
| --- | --- | --- | --- |
| DC_25A_n41A | DC_25A_n41A | No |  |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. |  |  |  |

### 5.18.2	Maximum output power

Table 5.18.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_25A_n41A | 266 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |  |  |

### 5.18.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Since MSD of CA_n25-n41 is already specified, it can be reused here

Table 5.18.3.-2: Reference sensitivity exceptions (MSD) due to cross band isolation and uplink/downlink configurations for PC2 EN-DC in NR FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n41 | 25 | 2546 | 100 | 30 | 270 (RBstart=0) | 1992.5 | 5 | 1.6 | >ACLR2 |

### 5.18.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 5.19	DC_25A_n78A

### 5.19.1	Configurations

Table 5.19.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed | DL interruption allowed (Note 14) |
| --- | --- | --- | --- |
| DC_25A_n78A | DC_25A_n78A | No |  |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. |  |  |  |

### 5.19.2	Maximum output power

Table 5.19.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_25A_n78A | 266 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |  |  |

### 5.19.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Since MSD of CA_n25-n78 is already specified, it can be reused here

Table 5.19.3-1: MSD test point for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_25A_n78A | 25 | 1855 | 5 | 25 | 1935 | 32.1 | IMD2 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 25 | 1855 | 5 | 25 | 1935 | 19.1 | IMD4 |
|  | n78 | 3690 | 10 | 50 | 3690 | N/A | N/A |
|  | 25 | 1875 | 5 | 25 | 1955 | 20 | IMD5 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |

Table 5.19.3.-2: Reference sensitivity exceptions (MSD) due to cross band isolation and uplink/downlink configurations for PC2 EN-DC in NR FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n78 | 25 | 3350 | 100 | 30 | 270 (RBstart=0) | 1992.5 | 5 | 1.0 | >ACLR2 |

### 5.19.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 5.20	DC_26A_n78A

### 5.20.1	Configurations

Table 5.20.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed | DL interruption allowed (Note 14) |
| --- | --- | --- | --- |
| DC_26A_n78A | DC_26A_n78A | No |  |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. |  |  |  |

### 5.20.2	Maximum output power

Table 5.20.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_26A_n78A | 266 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |  |  |

### 5.20.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Since MSD of CA_n26-n78 is already specified, it can be reused here.

Table 5.20.3-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for PC2 EN-DC in NR FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | 26 | 10 | 15 | 25 | 5 | 8.1 | NOTE 14 | UL1/DL4 |
| NOTE 14: The requirements should be verified for DL EARFCN or NR ARFCN of the victim (lower) band (superscript LB) such that and with the DL carrier frequency in the lower band and the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |

Table 5.20.3-2: MSD test point for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_26A_n78A | 26 | 836.5 | 5 | 25 | 881.5 | 23.8 | IMD4 |
|  | n78 | 3391 | 10 | 50 | 3391 | N/A | N/A |

### 5.20.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 5.21	DC_40A_n41A

### 5.21.1	Configurations

Table 5.21.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed | DL interruption allowed (Note 14) |
| --- | --- | --- | --- |
| DC_40A_n41A | DC_40A_n41A | No |  |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. |  |  |  |

### 5.21.2	Maximum output power

Table 5.21.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_40A_n41A | 266 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |  |  |

### 5.21.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Since MSD of CA_n40-n41 is already specified, it can be reused here

Table 5.21.3.-1: Reference sensitivity exceptions (MSD) due to cross band isolation and uplink/downlink configurations for PC2 EN-DC in NR FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n41 | 40 | 2546 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | 34.4 | ACLR2 |

### 5.21.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

# 6	High power UE (power class 1.5) for EN-DC

# Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-08 | RAN4 #112 | R4-2411281 |  |  |  | TR skeleton | 0.0.1 |
| 2024-08 | RAN4 #112 | R4-2411286 |  |  |  | Included TPs/pCRs: R4-2414391 TP for TR37.898 to add HP-ENDC 3-11_n79 and 8-42_n79 R4-2411261 TP for TR 37.898 to include DC_1A_n78A with 3Tx R4-2414442 TP for TR 37.898 to include DC_5A_n78A with 3Tx | 0.1.0 |
| 2024-10 | RAN4#112-bis | R4-2415259 |  |  |  | Included TPs/pCRs: R4-2415399 TP to TR 38.898 Addition of PC2 for DC_(n)40AA R4-2415793 TP for TR 37.898 to add of PC2 for DC_4A_n41A R4-2415794 TP for TR 37.898 to add of PC2 for DC_4A_n78A R4-2415795 TP for TR 37.898 to add of PC2 for DC_7C_n78A R4-2417055 TP for TR 37.898 to add of PC2 for DC_8A_n41A R4-2415797 TP for TR 37.898 to add of PC2 for DC_20A_n41A R4-2415798 TP for TR 37.898 to add of PC2 for DC_20A_n78A R4-2415799 TP for TR 37.898 to add of PC2 for DC_38A_n78A R4-2417056 TP for TR 37.898 to add of PC2 for DC_41A_n78A and DC_41C_n78A | 0.2.0 |
| 2025-02 | RAN4#114 | R4-2500386 |  |  |  | Included TPs/pCRs: R4-2500718 TP for TR 37.898 to add of PC2 for DC_11A_n41A R4-2500719 TP for TR 37.898 to add of PC2 for DC_12A_n41A R4-2502826 TP for TR 37.898 to add of PC2 for DC_12A_n78A R4-2503023 TP for TR 37.898 to add of PC2 for DC_18A_n78A R4-2502827 TP for TR 37.898 to add of PC2 for DC_25A_n41A R4-2502828 TP for TR 37.898 to add of PC2 for DC_25A_n78A R4-2503024 TP for TR 37.898 to add of PC2 for DC_26A_n78A R4-2500725 TP for TR 37.898 to add of PC2 for DC_40A_n41A | 0.3.0 |