| 3GPP TS 38.133 V18.0.0 (2022-12) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for support of radio resource management (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	82

1	Scope	84

2	References	84

3	Definitions, symbols and abbreviations	85

3.1	Definitions	85

3.2	Symbols	86

3.3	Abbreviations	87

3.4	Test tolerances	90

3.5	Frequency bands grouping	90

3.5.1	Introduction	90

3.5.2	NR operating bands in FR1	90

3.5.2A	NR operating bands for satellite access in FR1	91

3.5.3	NR operating bands in FR2	92

3.6	Applicability of requirements in this specification version	92

3.6.1	RRC connected state requirements in DRX	93

3.6.2	Number of serving carriers	93

3.6.2.1	Number of serving carriers for SA	93

3.6.2.2	Number of serving carriers for EN-DC	93

3.6.2.3	Number of serving carriers for NE-DC	93

3.6.2.4	Number of serving carriers for NR-DC	94

3.6.3	Applicability for intra-band FR2	94

3.6.4	Applicability for FR2 UE power classes	94

3.6.5	Applicability for SDL bands	94

3.6.6	Applicability of requirements for NGEN-DC operation	94

3.6.7	Applicability of QCL	94

3.6.9	Applicability of requirements for scheduling availability	95

3.6.10	Applicability of requirements for measurement restrictions	95

3.6.11	Applicability of requirements for Redcap UEs	95

3.6.11.1	RRC connected state requirements in DRX	95

3.6.11.2	Applicability for FR2 Redcap UE power classes	95

3.6.11.3	Applicability of QCL	95

3.6.12	Applicability of requirements for Satellite Access	96

3.6.13	Applicability of requirements for FR2	96

3.6.14	Applicability of requirements for FR2 Power Class 6	96

4	SA: RRC_IDLE state mobility	96

4.1	Cell Selection	96

4.2	Cell Re-selection	96

4.2.1	Introduction	96

4.2.2	Requirements	97

4.2.2.1	UE measurement capability	97

4.2.2.2	Measurement and evaluation of serving cell	97

4.2.2.3	Measurements of intra-frequency NR cells	98

4.2.2.4	Measurements of inter-frequency NR cells	100

4.2.2.5	Measurements of inter-RAT E-UTRAN cells	102

4.2.2.6	Maximum interruption in paging reception	104

4.2.2.7	General requirements	104

4.2.2.8	Minimum requirement at transitions	104

4.2.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	105

4.2.2.9.1	Introduction	105

4.2.2.9.2	Measurements for UE fulfilling low mobility criterion	105

4.2.2.9.3	Measurements for UE fulfilling not-at-cell edge criterion	106

4.2.2.9.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	107

4.2.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	107

4.2.2.10.1	Introduction	107

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion	107

4.2.2.10.3	Measurements for UE fulfilling not-at-cell edge criterion	108

4.2.2.10.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	109

4.2.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	110

4.2.2.11.1	Introduction	110

4.2.2.11.2	Measurements for UE fulfilling low mobility criterion	110

4.2.2.11.3	Measurements for UE fulfilling with not-at-cell edge criterion	111

4.2.2.11.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	112

4.2A	Cell Re-selection when subject to CCA	112

4.2A.1	Introduction	112

4.2A.2	Requirements	113

4.2A.2.1	UE measurement capability	113

4.2A.2.2	Measurement and evaluation when subject to CCA on the serving cell	113

4.2A.2.3	Measurements of intra-frequency NR cells when subject to CCA on the serving cell and target cell	114

4.2A.2.4	Measurements of inter-frequency NR cells when subject to CCA on the target cell	115

4.2A.2.5	Measurements of inter-RAT E-UTRAN cells when subject to CCA on the serving cell	117

4.2A.2.6	Maximum interruption in paging reception when subject to CCA on the target cell	117

4.2A.2.7	General requirements	118

4.2B	Cell Re-selection for RedCap	118

4.2B.1	Introduction	118

4.2B.2	Requirements	118

4.2B.2.1	UE measurement capability for RedCap	118

4.2B.2.1.1	UE measurement capability for 1 Rx RedCap	118

4.2B.2.1.2	UE measurement capability for 2 Rx RedCap	118

4.2B.2.2	Measurement and evaluation of serving cell for RedCap UE	118

4.2B.2.3	Measurements of intra-frequency NR cells for RedCap UE	121

4.2B.2.4	Measurements of inter-frequency NR cells for RedCap UE	123

4.2B.2.5	Measurements of inter-RAT E-UTRAN cells for RedCap UE	126

4.2B.2.6	Maximum interruption in paging reception for RedCap	128

4.2B.2.7	General requirements for RedCap	128

4.2B.2.8	Minimum requirement at transitions	129

4.2B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion for RedCap	129

4.2B.2.9.1	Introduction	129

4.2B.2.9.2	Measurements for UE fulfilling stationary criterion	130

4.2B.2.9.3	Measurements for a UE fulfilling stationary and not-at-cell edge criteria	133

4.2B.2.9.3A	Measurements for a UE fulfilling stationary and Rel-16 not at cell edge criteria	133

4.2B.2.9.4	Measurements for a UE fulfilling low mobility and stationary criteria	134

4.2B.2.9.5	Measurements for a UE fulfilling low mobility criterion and stationary and not-at-cell edge criteria	134

4.2B.2.9.6	Measurements for a UE fulfilling not-at-cell edge criterion and stationary and not-at-cell edge criteria	134

4.2B.2.9.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and stationary and notatcell edge criteria	135

4.2B.2.9.8	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and stationary criterion	135

4.2B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	135

4.2B.2.10.1	Introduction	135

4.2B.2.10.2	Measurements for UE fulfilling stationary criterion	136

4.2B.2.10.3	Measurements for a UE fulfilling stationary not at cell edge criterion	138

4.2B.2.10.3A	Measurements for a UE fulfilling stationary and Rel-16 not at cell edge criterion	139

4.2B.2.10.4	Measurements for a UE fulfilling low mobility and stationary criteria	139

4.2B.2.10.5	Measurements for a UE fulfilling low mobility and stationary not at cell edge criteria	140

4.2B.2.10.6	Measurements for a UE fulfilling not-at-cell edge criterion and stationary not at cell edge criteria	140

4.2B.2.10.7	Measurements for a UE fulfilling low mobility not-at-cell edge criterion and stationary not at cell edge criteria	140

4.2B.2.10.8	Measurements for a UE fulfilling low mobility not-at-cell edge criterion and stationary criteria	140

4.2B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	141

4.2B.2.11.1	Introduction	141

4.2B.2.11.2	Measurements for UE fulfilling stationary criterion	141

4.2B.2.11.3	Measurements for a UE fulfilling stationary not at cell edge criterion	143

4.2B.2.11.3A	Measurements for a UE fulfilling stationary and Rel-16 not at cell edge criterion	143

4.2B.2.11.4	Measurements for a UE fulfilling low mobility and stationary criteria	144

4.2B.2.11.5	Measurements for a UE fulfilling low mobility and stationary not at cell edge criteria	144

4.2B.2.11.6	Measurements for a UE fulfilling not-at-cell edge criterion and stationary not at cell edge criteria	144

4.2B.2.11.7	Measurements for a UE fulfilling low mobility not-at-cell edge criterion and stationary not at cell edge criteria	144

4.2B.2.11.8	Measurements for a UE fulfilling low mobility not-at-cell edge criterion and stationary criteria	145

4.2C	Cell Re-selection for NR UE for Satellite Access	145

4.2C.1	Introduction	145

4.2C.2	Requirements	145

4.2C.2.1	UE measurement capability	145

4.2C.2.2	Measurement and evaluation of serving cell	145

4.2C.2.3	Measurements of intra-frequency NR cells	146

4.2C.2.4	Measurements of inter-frequency NR cells	148

4.2C.2.5	Maximum interruption in paging reception	151

4.2C.2.6	Minimum requirement at transitions	152

4.2C.2.7	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	152

4.2C.2.8	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	152

4.2C.2.9	General requirements	152

4.3	Minimization of Drive Tests (MDT)	152

4.3.1	Introduction	152

4.3.2	Measurement Requirements	153

4.3.3	Requirements for Relative Time Stamp Accuracy	153

4.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	153

4.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	153

4.3C	Minimization of Drive Tests (MDT) for Satellite Access	154

4.3C.1	Introduction	154

4.3C.2	Measurement Requirements	154

4.3C.3	Requirements for Relative Time Stamp Accuracy	155

4.3C.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	155

4.3C.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	155

4.4	Idle Mode CA/DC Measurements	155

4.4.1	Introduction	155

4.4.2	Measurement Requirements	156

4.4.2.1	Detected cell requirement during state transition and Idle mode	156

4.4.2.2	Measurements of inter-frequency CA/DC candidate cells	156

4.4.2.3	Measurements on serving cell	157

4.4.2.4	Measurements of E-UTRAN inter-RAT DC candidate cells	158

5	SA: RRC_INACTIVE state mobility	159

5.1	Cell Re-selection	159

5.1.1	Introduction	159

5.1.2	Requirements	159

5.1.2.1	UE measurement capability	159

5.1.2.2	Measurement and evaluation of serving cell	159

5.1.2.3	Measurements of intra-frequency NR cells	159

5.1.2.4	Measurements of inter-frequency NR cells	159

5.1.2.5	Measurements of inter-RAT E-UTRAN cells	159

5.1.2.6	Maximum interruption in paging reception	159

5.1.2.7	General requirements	160

5.1A	Cell Re-selection with CCA	160

5.1A.1	Introduction	160

5.1A.2	Requirements	160

5.1A.2.1	UE measurement capability	160

5.1A.2.2	Measurement and evaluation when CCA is used on the serving cell	160

5.1A.2.3	Measurements of intra-frequency NR cells when CCA is used on the serving cell and target cell	160

5.1A.2.4	Measurements of inter-frequency NR cells when CCA is used on the target cell	160

5.1A.2.5	Measurements of inter-RAT E-UTRAN cells when CCA is used on the serving cell	160

5.1A.2.6	Maximum interruption in paging reception when CCA is used on the target cell	160

5.1A.2.7	General requirements	160

5.1B	Cell Re-selection for RedCap	161

5.1B.1	Introduction	161

5.1B.2	Requirements	161

5.1B.2.1	UE measurement capability	161

5.1B.2.2	Measurement and evaluation of serving cell	161

5.1B.2.3	Measurements of intra-frequency NR cells	162

5.1B.2.4	Measurements of inter-frequency NR cells	163

5.1B.2.5	Measurements of inter-RAT E-UTRAN cells	163

5.1B.2.6	Maximum interruption in paging reception	163

5.1B.2.7	General requirements	163

5.1B.2.8	Minimum requirement at transitions	163

5.1B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	164

5.1B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	164

5.1B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	164

5.2	Void	164

5.2B	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	164

5.2B.1	Introduction	164

5.2B.2	Requirements on UE synchronization for small data transmissions for RedCap	164

5.2B.2.2	Scheduling restriction	165

5.3	Minimization of Drive Tests (MDT)	165

5.3.1	Introduction	165

5.3.2	Measurement Requirements	166

5.3.3	Requirements for Relative Time Stamp Accuracy	166

5.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	166

5.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	166

5.3.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	166

5.3C	Minimization of Drive Tests (MDT) for Satellite Access	166

5.3C.1	Introduction	166

5.3C.2	Measurement Requirements	167

5.3C.3	Requirements for Relative Time Stamp Accuracy	167

5.3C.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	167

5.3C.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	167

5.3C.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	167

5.4	Idle Mode CA/DC Measurements	167

5.4.1	Introduction	167

5.4.2	Measurement Requirements	167

5.4.2.1	Detected cell requirement during state transition and Idle mode	168

5.4.2.2	Measurements of inter-frequency CA/DC candidate cells	168

5.4.2.3	Measurements on serving cell	168

5.4.2.4	Measurements on E-UTRAN inter-RAT DC candidate cells	168

5.5	Configured Grant based Small Data Transmissions (CG-SDT)	168

5.5.1	Introduction	168

5.5.2	Requirements on UE synchronization for small data transmissions	168

5.5.3	TA validation requirements	168

5.5.4	Scheduling restriction	169

5.5.4.1	Scheduling availability of UE performing measurements in TDD bands on FR1	169

5.5.4.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	169

5.5.4.3	Scheduling availability of UE performing measurements on FR2	170

5.5.5	Applicability conditions for SDT	170

5.6	NR measurements for positioning	170

5.6.1	Introduction	170

5.6.2	RSTD measurements	171

5.6.2.1	Introduction	171

5.6.2.2	Requirements Applicability	171

5.6.2.3	Measurement Capability	171

5.6.2.5	Measurements Period Requirements	172

5.6.3	PRS-RSRP measurements	175

5.6.3.1	Introduction	175

5.6.3.2	Requirements applicability	175

5.6.3.3	Measurement Capability	175

5.6.3.4	Measurement Reporting Requirements	175

5.6.3.5	Measurement Period Requirements	175

5.6.4	UE Rx-Tx time difference measurements	178

5.6.4.1	Introduction	178

5.6.4.2	Requirements Applicability	178

5.6.4.3	Measurement Capability	178

5.6.4.4	Measurement Reporting Requirements	178

5.6.4.5	Measurement Period Requirements	178

5.6.5	PRS-RSRPP measurements	181

5.6.5.1	Introduction	181

5.6.5.2	Requirements applicability	181

5.6.5.3	Measurement capability	181

5.6.5.4	Measurement reporting requirements	181

5.6.5.5	Measurement period requirements	182

5.7	Random access based Small Data Transmissions (RA-SDT)	182

5.7.1	Introduction	182

5.7.2	Requirements for small data transmissions based on 2-step RA	182

5.7.3	Requirements for small data transmissions based on 4-step RA	182

6	RRC_CONNECTED state mobility	182

6.1	Handover	182

6.1.1	NR Handover	182

6.1.1.1	Introduction	182

6.1.1.2	NR FR1 - NR FR1 Handover	182

6.1.1.2.1	Handover delay	183

6.1.1.2.2	Interruption time	183

6.1.1.3	NR FR2- NR FR1 Handover	183

6.1.1.3.1	Handover delay	183

6.1.1.3.2	Interruption time	184

6.1.1.4	NR FR2- NR FR2 Handover	184

6.1.1.4.1	Handover delay	184

6.1.1.4.2	Interruption time	184

6.1.1.5	NR FR1- NR FR2 Handover	185

6.1.1.5.1	Handover delay	185

6.1.1.5.2	Interruption time	185

6.1.2	NR Handover to other RATs	186

6.1.2.1	NR – E-UTRAN Handover	186

6.1.2.1.1	Introduction	186

6.1.2.1.2	Handover delay	186

6.1.2.1.3	Interruption time	187

6.1.2.2	NR – UTRAN Handover	187

6.1.2.2.1	Introduction	187

6.1.2.2.2	Handover delay	187

6.1.2.2.3	Interruption time	187

6.1.3	NR DAPS Handover	188

6.1.3.1	Introduction	188

6.1.3.2	NR FR1 - NR FR1 DAPS Handover	188

6.1.3.2.1	DAPS handover delay	189

6.1.3.2.2	Interruption time	189

6.1.3.3	NR FR2- NR FR1 DAPS Handover	191

6.1.3.3.1	DAPS handover delay	191

6.1.3.3.2	Interruption time	192

6.1.3.4	NR FR1- NR FR2 DAPS Handover	192

6.1.3.4.1	DAPS handover delay	193

6.1.3.4.2	Interruption time	193

6.1.4	NR Conditional Handover	193

6.1.4.1	Introduction	193

6.1.4.2	NR FR1 – NR FR1 conditional handover	194

6.1.4.3	NR FR2 – NR FR1 conditional handover	195

6.1.4.4	NR FR2 – NR FR2 conditional handover	195

6.1.4.4.1	Handover delay	195

6.1.4.4.2	Measurement time	195

6.1.4.4.3	Preparation time	196

6.1.4.4.4	Interruption time	196

6.1.4.5	NR FR1 – NR FR2 conditional handover	196

6.1.5	NR Handover with PSCell	197

6.1.5.1	Introduction	197

6.1.5.2	Handover with PSCell from NR SA to EN-DC	197

6.1.5.2.1	Interruption time for inter-RAT HO from NR to E-UTRAN	197

6.1.5.2.2	PSCell addition in HO with PSCell for NR SA to EN-DC	197

6.1.5.3	HO with PSCell from NE-DC to NE-DC	198

6.1.5.3.1	Handover delay	198

6.1.5.3.2	HO with PSCell - PCell Interruption time	198

6.1.5.3.3	PSCell addition/change in NE-DC to NE-DC HO with PSCell	199

6.1.5.4	HO with PSCell from NR-DC to NR-DC	199

6.1.5.5	Handover with PSCell from NR SA to EN-DC with PSCell using CCA	200

6.1.5.5.1	Introduction	200

6.1.5.5.2	NR SA to EN-DC HO with PSCell- NR to E-UTRA HO Interruption time	200

6.1.5.5.3	NR SA to EN-DC HO with PSCell - NR PSCell Addition Delay requirements	201

6.1A	Void	202

6.1A.1	Void	202

6.1A.1.1	Void	202

6.1A.1.2	Void	202

6.1A.1.2.1	Void	202

6.1A.1.2.2	Void	202

6.1B	Handover to target cell using CCA	202

6.1B.1	NR Handover	202

6.1B.1.1	Introduction	202

6.1B.1.2	NR FR1 - NR FR1 Handover	202

6.1B.1.2.1	Handover delay	202

6.1B.1.2.2	Interruption time	203

6.1B.1.3	NR FR2-2 NR FR2-2 Handover	203

6.1B.1.3.1	Handover delay	203

6.1B.1.3.2	Interruption time	204

6.1B.1.4	NR FR1- NR FR2-2 Handover	204

6.1B.1.4.1	Handover delay	205

6.1B.1.4.2	Interruption time	205

6.1C	Handover for SAN	206

6.1C.1	NR SAN Handover	206

6.1C.1.1	Introduction	206

6.1C.1.2	NR SAN FR1 – NR SAN FR1 Handover	206

6.1C.1.2.1	Handover delay	206

6.1C.1.2.2	Interruption time	206

6.1C.2	NR SAN Conditional Handover	207

6.1C.2.1	Introduction	207

6.1C.2.2	NR SAN FR1 – NR SAN FR1 conditional handover	207

6.1C.2.2.1	Handover delay	207

6.1C.2.2.2	Measurement time	208

6.1C.2.2.3	Preparation time	209

6.1C.2.2.4	Interruption time	209

6.1D	Handover for RedCap	209

6.1D.1	NR Handover	209

6.1D.1.1	Introduction	209

6.1D.1.2	NR FR1 - NR FR1 Handover	210

6.1D.1.3	NR FR2- NR FR2 Handover	210

6.1D.2	NR Handover to other RATs	210

6.1D.2.1	NR – E-UTRAN Handover	210

6.2	RRC Connection Mobility Control	211

6.2.1	SA: RRC Re-establishment	211

6.2.1.1	Introduction	211

6.2.1.2	Requirements	211

6.2.1.2.1	UE Re-establishment delay requirement	211

6.2.1A	RRC Re-establishment with CCA	213

6.2.1A.1	Introduction	213

6.2.1A.2	Requirements	213

6.2.1A.2.1	UE Re-establishment with CCA delay requirement	213

6.2.1B	SA: RRC Re-establishment for RedCap	215

6.2.1B.1	Introduction	215

6.2.1B.2	Requirements	215

6.2.2	Random access	215

6.2.2.1	Introduction	215

6.2.2.2	Requirements for 4-step RA type	216

6.2.2.2.1	Contention based random access	216

6.2.2.2.1.1	Correct behaviour when transmitting Random Access Preamble	216

6.2.2.2.1.2	Correct behaviour when receiving Random Access Response	216

6.2.2.2.1.3	Correct behaviour when not receiving Random Access Response	216

6.2.2.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	216

6.2.2.2.1.5	SA: Correct behaviour when receiving a message over Temporary C-RNTI	217

6.2.2.2.1.6	Correct behaviour when contention Resolution timer expires	217

6.2.2.2.2	Non-Contention based random access	217

6.2.2.2.2.1	Correct behaviour when transmitting Random Access Preamble	217

6.2.2.2.2.2	Correct behaviour when receiving Random Access Response	217

6.2.2.2.2.3	Correct behaviour when not receiving Random Access Response	218

6.2.2.2.3	UE behaviour when configured with supplementary UL	218

6.2.2.3	Requirements for 2-step RA type	218

6.2.2.3.1	Contention based random access	218

6.2.2.3.1.1	Correct behaviour when transmitting MsgA	218

6.2.2.3.1.2	Correct behaviour when receiving MsgB	219

6.2.2.3.1.3	Correct behaviour when not receiving MsgB	219

6.2.2.3.2	Non-Contention based random access	219

6.2.2.3.2.1	Correct behaviour when transmitting MsgA	219

6.2.2.3.2.2	Correct behaviour when receiving MsgB	219

6.2.2.3.2.3	Correct behaviour when not receiving MsgB	219

6.2.2.3.3	UE behaviour when configured with supplementary UL	220

6.2.2A	Random access when CCA is used on target frequency	220

6.2.2A.1	Introduction	220

6.2.2A.2	Requirements for 4-step RA type	220

6.2.2A.2.1	Contention based random access	220

6.2.2A.2.1.1	Correct behaviour when transmitting Random Access Preamble	220

6.2.2A.2.1.2	Correct behaviour when receiving Random Access Response	221

6.2.2A.2.1.3	Correct behaviour when not receiving Random Access Response	221

6.2.2A.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	221

6.2.2A.2.1.6	Correct behaviour when contention Resolution timer expires	221

6.2.2A.2.2	Non-Contention based random access	221

6.2.2A.2.2.1	Correct behaviour when transmitting Random Access Preamble	221

6.2.2A.2.2.2	Correct behaviour when receiving Random Access Response	222

6.2.2A.2.2.3	Correct behaviour when not receiving Random Access Response	222

6.2.2A.3	Requirements for 2-step RA type	222

6.2.2A.3.1	Contention based random access	223

6.2.2A.3.1.1	Correct behaviour when transmitting MsgA	223

6.2.2A.3.1.2	Correct behaviour when receiving MsgB	223

6.2.2A.3.1.3	Correct behaviour when not receiving MsgB	224

6.2.2A.3.2	Non-Contention based random access	224

6.2.2A.3.2.1	Correct behaviour when transmitting MsgA	224

6.2.2A.3.2.2	Correct behaviour when receiving MsgB	224

6.2.2A.3.2.3	Correct behaviour when not receiving MsgB	224

6.2.2B	Random access for RedCap	225

6.2.2B.1	Introduction	225

6.2.2B.2	Requirements	225

6.2.3	SA: RRC Connection Release with Redirection	225

6.2.3.1	Introduction	225

6.2.3.2	Requirements	226

6.2.3.2.1	RRC connection release with redirection to NR	226

6.2.3.2.2	RRC connection release with redirection to E-UTRAN	226

6.2.3.2.3	RRC connection release with redirection to NR carrier subject to CCA	227

6.2.3A	SA: RRC Connection Release with Redirection for RedCap	228

6.2.3A.1	Introduction	228

6.2.3A.2	Requirements	228

6.2.3A.2.1	RRC connection release with redirection to NR	228

6.2.3A.2.2	RRC connection release with redirection to E-UTRAN	229

6.2C	RRC Connection Mobility Control for Satellite Access	229

6.2C.1	SA: RRC Re-establishment for Satellite Access	229

6.2C.1.1	Introduction	229

6.2C.1.2	Requirements	229

6.2C.1.2.1	UE Re-establishment delay requirement	229

6.2C.2	Random access for satellite access	230

6.2C.2.1	Introduction	230

6.2C.2.2	Requirements for 4-step RA type	231

6.2C.2.2.1	Contention based random access	231

6.2C.2.2.1.1	Correct behaviour when transmitting Random Access Preamble	231

6.2C.2.2.1.2	Correct behaviour when receiving Random Access Response	231

6.2C.2.2.1.3	Correct behaviour when not receiving Random Access Response	231

6.2C.2.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	231

6.2C.2.2.1.5	SA: Correct behaviour when receiving a message over Temporary C-RNTI	231

6.2C.2.2.1.6	Correct behaviour when contention Resolution timer expires	232

6.2C.2.2.2	Non-Contention based random access	232

6.2C.2.2.2.1	Correct behaviour when transmitting Random Access Preamble	232

6.2C.2.2.2.2	Correct behaviour when receiving Random Access Response	232

6.2C.2.2.2.3	Correct behaviour when not receiving Random Access Response	233

6.2C.2.3	Requirements for 2-step RA type	233

6.2C.2.3.1	Contention based random access	233

6.2C.2.3.1.1	Correct behaviour when transmitting MsgA	233

6.2C.2.3.1.2	Correct behaviour when receiving MsgB	233

6.2C.2.3.1.3	Correct behaviour when not receiving MsgB	234

6.2C.2.3.2	Non-Contention based random access	234

6.2C.2.3.2.1	Correct behaviour when transmitting MsgA	234

6.2C.2.3.2.2	Correct behaviour when receiving MsgB	234

6.2C.2.3.2.3	Correct behaviour when not receiving MsgB	234

6.2C.3	SA: RRC Connection Release with Redirection for Satellite Access	234

6.2C.3.1	Introduction	234

6.2C.3.2	Requirements	235

6.2C.3.2.1	RRC connection release with redirection to NR	235

7	Timing	236

7.1	UE transmit timing	236

7.1.1	Introduction	236

7.1.2	Requirements	236

7.1.2.1	Gradual timing adjustment	238

7.1.2.2	Void	238

7.1.2.3	One shot large UL timing adjustment for FR2 Power Class 6 UE	238

7.1A	UE transmit timing for RedCap	239

7.1A.1	Introduction	239

7.1A.2	Requirements	239

7.1A.2.1	Gradual timing adjustment	240

7.1C	UE transmit timing for Satellite Access	241

7.1C.1	Introduction	241

7.1C.2	Requirements	241

7.1C.2.1	Gradual timing adjustment	242

7.2	UE timer accuracy	242

7.2.1	Introduction	242

7.2.2	Requirements	242

7.2A	UE timer accuracy for RedCap	243

7.2A.1	Introduction	243

7.2A.2	Requirements	243

7.2C	UE timer accuracy for satellite access	243

7.2C.1	Introduction	243

7.2C.2	Requirements	243

7.3	Timing advance	244

7.3.1	Introduction	244

7.3.2	Requirements	244

7.3.2.1	Timing Advance adjustment delay	244

7.3.2.2	Timing Advance adjustment accuracy	244

7.3A	Timing Advance for RedCap	244

7.3A.1	Introduction	244

7.3A.2	Requirements	244

7.3A.2.1	Timing Advance adjustment delay	244

7.3A.2.2	Timing Advance adjustment accuracy	244

7.3C	Timing advance for satellite access	245

7.3C.1	Introduction	245

7.3C.2	Requirements	245

7.3C.2.1	Timing Advance adjustment delay	245

7.3C.2.2	Timing Advance adjustment accuracy	245

7.4	Cell phase synchronization accuracy	245

7.4.1	Definition	245

7.4.2	Minimum requirements	245

7.5	Maximum Transmission Timing Difference	246

7.5.1	Introduction	246

7.5.2	Minimum Requirements for inter-band EN-DC	246

7.5.2.1	Minimum Requirements for inter-band synchronous EN-DC	246

7.5.3	Minimum Requirements for intra-band EN-DC	247

7.5.4	Minimum Requirements for NR Carrier Aggregation	247

7.5.5	Minimum Requirements for inter-band NE-DC	248

7.5.5.1	Minimum Requirements for inter-band synchronous NE-DC	248

7.5.6	Minimum Requirements for inter-band NR DC	248

7.6	Maximum Receive Timing Difference	249

7.6.1	Introduction	249

7.6.2	Minimum Requirements for inter-band EN-DC	250

7.6.2.1	Minimum Requirements for inter-band synchronous EN-DC	250

7.6.3	Minimum Requirements for intra-band EN-DC	250

7.6.4	Minimum Requirements for NR Carrier Aggregation	251

7.6.5	Minimum Requirements for inter-band NE-DC	251

7.6.5.1	Minimum Requirements for inter-band synchronous NE-DC	252

7.6.6	Minimum Requirements for inter-band NR DC	252

7.7	deriveSSB-IndexFromCell tolerance	253

7.7.1	Minimum requirements	253

7.7A	deriveSSB-IndexFromCell tolerance for RedCap	254

7.7A.1	Minimum requirements	254

7.8	Void	254

7.9	deriveSSB-IndexFromCellInter-r17 tolerance	254

7.9.1	Minimum requirements	254

8	Signalling characteristics	254

8.1	Radio Link Monitoring	254

8.1.1	Introduction	254

8.1.1.1	Introduction of Requirement on Radio Link Monitoring for UE Configured with Relaxed Measurement Criteria	255

8.1.2	Requirements for SSB based radio link monitoring	256

8.1.2.1	Introduction	256

8.1.2.2	Minimum requirement	257

8.1.2.3	Measurement restrictions for SSB based RLM	260

8.1.2.4	Minimum requirement of SSB based radio link monitoring for UE fulfilling relaxed measurement criteria	261

8.1.3	Requirements for CSI-RS based radio link monitoring	262

8.1.3.1	Introduction	262

8.1.3.2	Minimum requirement	263

8.1.3.3	Measurement restrictions for CSI-RS based RLM	266

8.1.3.4	Minimum requirement of CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria	267

8.1.4	Minimum requirement at transitions	267

8.1.5	Minimum requirement for UE turning off the transmitter	268

8.1.6	Minimum requirement for L1 indication	268

8.1.7	Scheduling availability of UE during radio link monitoring	268

8.1.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	268

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	269

8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2	269

8.1.7.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	270

8.1A	Radio Link Monitoring with CCA on Target Frequency	270

8.1A.1	Introduction	270

8.1A.2	Requirements for SSB Based Radio Link Monitoring	271

8.1A.2.1	Introduction	271

8.1A.2.2	Minimum Requirement	271

8.1A.3	Minimum requirement at transitions	275

8.1A.4	Minimum requirement for UE turning off the transmitter	275

8.1A.5	Minimum requirement for L1 indication	275

8.1A.6	Scheduling availability of UE during radio link monitoring	275

8.1A.6.3	Scheduling availability of UE performing radio link monitoring on FR2-2	276

8.1A.6.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	277

8.1B	Radio Link Monitoring for RedCap	277

8.1B.1	Introduction	277

8.1B.2	Requirements for SSB based radio link monitoring	278

8.1B.2.1	Introduction	278

8.1B.2.2	Minimum requirement	279

8.1B.2.3	Measurement restrictions for SSB based RLM	281

8.1B.3	Requirements for CSI-RS based radio link monitoring	281

8.1B.3.1	Introduction	281

8.1B.3.2	Minimum requirement	282

8.1B.3.3	Measurement restrictions for CSI-RS based RLM	285

8.1B.4	Minimum requirement at transitions	285

8.1B.5	Minimum requirement for UE turning off the transmitter	286

8.1B.6	Minimum requirement for L1 indication	286

8.1B.7	Scheduling availability of UE during radio link monitoring	286

8.1B.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	286

8.1B.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	286

8.1B.7.3	Scheduling availability of UE performing radio link monitoring on FR2	287

8.1C	Radio Link Monitoring for Satellite Access	287

8.1C.1	Introduction	287

8.1C.2	Requirements for SSB based radio link monitoring	288

8.1C.2.1	Introduction	288

8.1C.2.2	Minimum requirement	289

8.1C.2.3	Measurement restrictions for SSB based RLM	290

8.1C.3	Requirements for CSI-RS based radio link monitoring	290

8.1C.3.1	Introduction	290

8.1C.3.2	Minimum requirement	291

8.1C.3.3	Measurement restrictions for CSI-RS based RLM	292

8.1C.4	Minimum requirement at transitions	292

8.1C.5	Minimum requirement for UE turning off the transmitter	293

8.1C.6	Minimum requirement for L1 indication	293

8.1C.7	Scheduling availability of UE during radio link monitoring	293

8.1C.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	293

8.1C.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	293

8.2	Interruption	294

8.2.1	EN-DC Interruption	294

8.2.1.1	Introduction	294

8.2.1.2	Requirements	295

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX	295

8.2.1.2.2	Interruptions at transitions from non-DRX to DRX	295

8.2.1.2.3	Interruptions at SCell addition/release	295

8.2.1.2.4	Interruptions at SCell activation/deactivation	296

8.2.1.2.5	Interruptions during measurements on SCC	297

8.2.1.2.5.1	Interruptions during measurements on deactivated NR SCC	297

8.2.1.2.5.2	Interruptions during measurements on deactivated E-UTRAN SCC	297

8.2.1.2.5.3	Interruptions during CQI measurements on dormant E-UTRAN SCell	298

8.2.1.2.5.4	Interruptions during RRM measurements on dormant E-UTRAN SCC	298

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration	299

8.2.1.2.7	Interruptions due to Active BWP switching Requirement	299

8.2.1.2.8	Interruptions at direct SCell activation and hibernation	300

8.2.1.2.8.1	Interruptions during direct SCell activation and hibernation of E-UTRA SCell	300

8.2.1.2.8.2	Interruptions during direct SCell activation	300

8.2.1.2.9	Interruptions at SCell hibernation	301

8.2.1.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	301

8.2.1.2.11	Interruptions due to UE-specific CBW change	301

8.2.1.2.12	Interruptions at NR SRS carrier based switching	301

8.2.1.2.13	Interruptions at E-UTRA SRS carrier based switching	303

8.2.1.2.14	DL Interruptions at switching between two uplink carriers	304

8.2.1.2.15	Interruptions due to SCell dormancy	304

8.2.1.2.15.1	Interruptions due to SCell dormancy switch	304

8.2.1.2.15.2	Interruptions due to CQI measurements during SCell dormancy	304

8.2.1.2.15.3	Interruptions due to RRM measurements during SCell dormancy	305

8.2.1.2.16	Interruptions when identifying CGI of an NR cell with autonomous gaps	305

8.2.1.2.17	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	305

8.2.1.2.18	Interruptions at NR SRS antenna port switching	306

8.2.1.2.19	Interruptions at fast SCell activation	307

8.2.1.2.20	Interruptions due to PUCCH SCell activation/deactivation	308

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation	308

8.2.2.1	Introduction	308

8.2.2.2	Requirements	309

8.2.2.2.1	Interruptions at SCell addition/release	309

8.2.2.2.2	Interruptions at SCell activation/deactivation	310

8.2.2.2.3	Interruptions during measurements on deactivated SCC	311

8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration	312

8.2.2.2.5	Interruptions due to Active BWP switching Requirement	312

8.2.2.2.6	Interruptions at inter-frequency SFTD measurement	313

8.2.2.2.7	Interruptions at SCell activation/deactivation with multiple downlink SCells	315

8.2.2.2.8	Interruptions due to UE-specific CBW change	315

8.2.2.2.9	Interruptions at NR SRS carrier based switching	315

8.2.2.2.10	DL Interruptions at UE switching between two uplink carriers	317

8.2.2.2.10A	DL Interruptions at UE switching between two uplink carriers with two transmit antenna connectors	317

8.2.2.2.10B	DL Interruptions at UE switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	318

8.2.2.2.10C	DL Interruptions at UE switching between two uplink bands with two transmit antenna connectors	318

8.2.2.2.11	Interruptions at direct SCell activation	318

8.2.2.2.12	Interruptions due to SCell dormancy	319

8.2.2.2.12.1	Interruptions due to SCell dormancy switch	319

8.2.2.2.12.2	Interruptions due to CQI measurements during SCell dormancy	319

8.2.2.2.12.3	Interruptions due to RRM measurements during SCell dormancy	319

8.2.2.2.13	Interruptions at transitions between active and non-active during DRX	319

8.2.2.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	319

8.2.2.2.15	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	320

8.2.2.2.16	Interruptions at NR SRS antenna port switching	320

8.2.2.2.17	Interruptions at fast SCell activation	321

8.2.2.2.18	Interruptions due to PUCCH SCell activation/deactivation	322

8.2.3	NE-DC Interruptions	322

8.2.3.1	Introduction	322

8.2.3.2	Requirements	323

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX	323

8.2.3.2.2	Interruptions at transitions from non-DRX to DRX	323

8.2.3.2.3	Interruptions at PSCell/SCell addition/release	323

8.2.3.2.4	Interruptions at SCell activation/deactivation	325

8.2.3.2.5	Interruptions during measurements on SCC	326

8.2.3.2.5.1	Interruptions during measurements on deactivated NR SCC	326

8.2.3.2.5.2	Interruptions during measurements on deactivated E-UTRAN SCC	326

8.2.3.2.5.3	Interruptions during CQI measurements on dormant E-UTRAN SCC	327

8.2.3.2.5.4	Interruptions during RRM measurements on dormant E-UTRAN SCC	327

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration	327

8.2.3.2.7	Interruptions due to Active BWP switching Requirement	328

8.2.3.2.8	Interruptions at direct SCell activation and hibernation	328

8.2.3.2.9	Interruptions at SCell hibernation	328

8.2.3.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	329

8.2.3.2.11	 Interruptions at NR SRS carrier based switching	329

8.2.3.2.12	 Interruptions at E-UTRA SRS carrier based switching	330

8.2.3.2.13	Interruptions due to SCell dormancy	331

8.2.3.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	332

8.2.3.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	332

8.2.3.2.17	Interruptions at fast SCell activation	334

8.2.3.2.18	Interruptions due to UE-specific CBW change	334

8.2.3.2.19	Interruptions due to PUCCH SCell activation/deactivation	335

8.2.4	NR-DC: Interruptions	335

8.2.4.1	Introduction	335

8.2.4.2	Requirements	336

8.2.4.2.1	Interruptions at PSCell/SCell addition/release	336

8.2.4.2.2	Interruptions at SCell activation/deactivation	337

8.2.4.2.3	Interruptions during measurements on SCC	338

8.2.4.2.4	Interruptions at UL carrier RRC reconfiguration	338

8.2.4.2.5	Interruptions due to Active BWP switching Requirement	339

8.2.4.2.6	Interruptions at transitions between active and non-active during DRX	339

8.2.4.2.7	Interruptions at transitions from non-DRX to DRX	339

8.2.4.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	340

8.2.4.2.9	Interruptions at NR SRS carrier based switching	340

8.2.4.2.10	Interruptions at direct SCell activation	341

8.2.4.2.11	Interruptions when identifying CGI of an NR cell with autonomous gaps	342

8.2.4.2.12	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	342

8.2.4.2.13	 Interruptions due to SCell dormancy	343

8.2.4.2.14	Interruptions at NR SRS antenna port switching	343

8.2.4.2.15	Interruptions at fast SCell activation	344

8.2.4.2.16	Interruptions at SCG activation/deactivation	345

8.2.4.2.17	Interruptions due to RRM measurements on deactivated SCG	345

8.2.4.2.18 Interruptions during RLM/BFD measurements on deactivated PScell	346

8.2.4.2.19	Interruptions due to UE-specific CBW change	346

8.2.4.2A	Void	346

8.2.4.2A.1	Void	346

8.2.4.2A.2	Void	346

8.2.4.2A.3	Void	346

8.3	SCell Activation and Deactivation Delay	346

8.3.1	Introduction	346

8.3.2	SCell Activation Delay Requirement for Deactivated SCell	346

8.3.3	SCell Deactivation Delay Requirement for Activated SCell	351

8.3.4	Direct SCell Activation at SCell addition	351

8.3.5	Direct SCell Activation at Handover	353

8.3.7	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	354

8.3.8	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	358

8.3.9	Direct SCell Activation of Multiple Downlink SCells at SCell addition	359

8.3.10	Direct SCell Activation of Multiple Downlink SCells at Handover	360

8.3.12	SCell Activation Delay Requirement for Deactivated PUCCH SCell	361

8.3.13	SCell activation delay Requirement for Deactivated PUCCH SCell with Multiple SCells	364

8.3.13.1	Introduction	364

8.3.14	SCell Deactivation Delay Requirement for Activated PUCCH SCell	366

8.3.15	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple Downlink SCells	366

8.3.16	Fast SCell Activation Delay Requirement for Deactivated SCell	367

8.3A	SCell Activation and Deactivation Delay in Carriers with CCA	369

8.3A.1	Introduction	369

8.3A.2	SCell Activation Delay Requirement for Deactivated SCell	370

8.3A.3	SCell Deactivation Delay Requirement for Activated SCell	374

8.4	UE UL carrier RRC reconfiguration delay	374

8.4.1	Introduction	374

8.4.2	UE UL carrier configuration delay requirement	374

8.4.3	UE UL carrier deconfiguration delay requirement	375

8.5	Link Recovery Procedures	375

8.5.1	Introduction	375

8.5.1.1	Introduction of Requirement on Link Recovery Procedures for UE configured with relaxed measurement criteria	376

8.5.2	Requirements for SSB based beam failure detection	377

8.5.2.1	Introduction	377

8.5.2.2	Minimum requirement	377

8.5.2.3	Measurement restriction for SSB based beam failure detection	380

8.5.2.4	Minimum requirement of SSB based beam failure detection for UE fulfilling relaxed measurement criteria	381

8.5.3	Requirements for CSI-RS based beam failure detection	382

8.5.3.1	Introduction	382

8.5.3.2	Minimum requirement	382

8.5.3.3	Measurement restrictions for CSI-RS beam failure detection	385

8.5.3.4	Minimum requirement of CSI-RS based beam failure detection for UE fulfilling relaxed measurement criteria	386

8.5.4	Minimum requirement for L1 indication	387

8.5.5	Requirements for SSB based candidate beam detection	388

8.5.5.1	Introduction	388

8.5.5.2	Minimum requirement	388

8.5.5.3	Measurement restriction for SSB based candidate beam detection	391

8.5.6	Requirements for CSI-RS based candidate beam detection	391

8.5.6.1	Introduction	391

8.5.6.2	Minimum requirement	391

8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection	394

8.5.7	Scheduling availability of UE during beam failure detection	395

8.5.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	395

8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	395

8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2	395

8.5.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	396

8.5.8	Scheduling availability of UE during candidate beam detection	396

8.5.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	396

8.5.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	396

8.5.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	397

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	397

8.5.9	Requirements for Beam Failure Recovery in SCell	398

8.5.9.1	Introduction	398

8.5.9.2	Requirement	398

8.5.10	Minimum requirement at transitions for beam failure detection	398

8.5A	Link Recovery Procedures when CCA is used on target frequency	398

8.5A.1	Introduction	398

8.5A.2	Requirements for SSB based beam failure detection	399

8.5A.2.1	Introduction	399

8.5A.2.2	Minimum requirement	400

8.5A.2.3	Measurement restriction for SSB based beam failure detection	402

8.5A.3	Void	402

8.5A.4	Minimum requirement for L1 indication	402

8.5A.5	Requirements for SSB based candidate beam detection	403

8.5A.5.1	Introduction	403

8.5A.5.2	Minimum requirement	403

8.5A.5.3	Measurement restriction for SSB based candidate beam detection	405

8.5A.6	Void	406

8.5A.7	Scheduling availability of UE during beam failure detection	406

8.5A.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH	406

8.5A.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH	406

8.5A.7.3	Scheduling availability of UE performing beam failure detection on FR2-2	406

8.5A.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR DC	406

8.5A.8	Scheduling availability of UE during candidate beam detection	406

8.5A.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2-2	406

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	406

8.5B	Link Recovery Procedures for Redcap	407

8.5B.1	Introduction	407

8.5B.2	Requirements for SSB based beam failure detection for Redcap	407

8.5B.2.1	Introduction	407

8.5B.2.2	Minimum requirement	408

8.5B.2.3	Measurement restriction for SSB based beam failure detection	409

8.5B.3	Requirements for CSI-RS based beam failure detection for Redcap	410

8.5B.3.1	Introduction	410

8.5B.3.2	Minimum requirement	410

8.5B.3.3	Measurement restrictions for CSI-RS beam failure detection	412

8.5B.4	Minimum requirement for L1 indication for Redcap	413

8.5B.5	Requirements for SSB based candidate beam detection for Redcap	414

8.5B.5.1	Introduction	414

8.5B.5.2	Minimum requirement	414

8.5B.5.3	Measurement restriction for SSB based candidate beam detection	416

8.5B.6	Requirements for CSI-RS based candidate beam detection for Redcap	416

8.5B.6.1	Introduction	416

8.5B.6.2	Minimum requirement	416

8.5B.6.3	Measurement restriction for CSI-RS based candidate beam detection	418

8.5B.7	Scheduling availability of UE during beam failure detection for Redcap	419

8.5B.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	419

8.5B.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	419

8.5B.7.3	Scheduling availability of UE performing beam failure detection on FR2	419

8.5B.8	Scheduling availability of UE during candidate beam detection for Redcap	420

8.5B.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	420

8.5B.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	420

8.5B.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	420

8.5B.9	Minimum requirement at transitions for beam failure detection for Redcap	420

8.5C	Link Recovery Procedures for Satellite Access	421

8.5C.1	Introduction	421

8.5C.2	Requirements for SSB based beam failure detection	421

8.5C.2.1	Introduction	421

8.5C.2.2	Minimum requirement	422

8.5C.2.3	Measurement restriction for SSB based beam failure detection	422

8.5C.3	Requirements for CSI-RS based beam failure detection	423

8.5C.3.1	Introduction	423

8.5C.3.2	Minimum requirement	423

8.5C.3.3	Measurement restrictions for CSI-RS beam failure detection	424

8.5C.4	Minimum requirement for L1 indication	425

8.5C.5	Requirements for SSB based candidate beam detection	425

8.5C.5.1	Introduction	425

8.5C.5.2	Minimum requirement	425

8.5C.5.3	Measurement restriction for SSB based candidate beam detection	426

8.5C.6	Requirements for CSI-RS based candidate beam detection	427

8.5C.6.1	Introduction	427

8.5C.6.2	Minimum requirement	427

8.5C.6.3	Measurement restriction for CSI-RS based candidate beam detection	428

8.5C.7	Scheduling availability of UE during beam failure detection	428

8.5C.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	428

8.5C.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	428

8.5C.8	Scheduling availability of UE during candidate beam detection	429

8.5C.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	429

8.5C.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	429

8.5C.9	Minimum requirement at transitions for beam failure detection	429

8.6	Active BWP switch delay	429

8.6.1	Introduction	429

8.6.2	DCI and timer based BWP switch delay on a single CC	430

8.6.2A	DCI based BWP switch delay on multiple CCs	431

8.6.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs	431

8.6.2A.2	Non-simultaneous DCI based BWP switch delay on multiple CCs	433

8.6.2B	Timer based BWP switch delay on multiple CCs	433

8.6.2B.1	Simultaneous timer based BWP switch delay on multiple CCs	433

8.6.2B.2	Non-simultaneous timer based BWP switch delay on multiple CCs	433

8.6.3	RRC based BWP switch delay on a single CC	434

8.6.3A	RRC based BWP switch delay on multiple CCs	435

8.6.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs	435

8.6.3A.2	Non-simultaneous RRC based BWP switch delay on multiple CCs	435

8.6.4	BWP switch delay on Consistent UL CCA recovery	436

8.6A	Active BWP switch delay for RedCap	436

8.6A.1	Introduction	436

8.6A.2	DCI and timer based BWP switch delay on a single CC	436

8.6A.3	RRC based BWP switch delay on a single CC	438

8.6C	Active BWP switch delay for satellite access	438

8.6C.1	Introduction	438

8.6C.2	DCI and timer based BWP switch delay on a single CC	438

8.6C.3	RRC based BWP switch delay on a single CC	440

8.7	Void	440

8.8	NE-DC: E-UTRAN PSCell Addition and Release Delay	440

8.8.1	Introduction	440

8.8.2	E-UTRAN PSCell Addition Delay Requirement	440

8.8.3	E-UTRAN PSCell Release Delay Requirement	441

8.9	NR-DC: PSCell Addition and Release Delay	441

8.9.1	Introduction	441

8.9.2	PSCell Addition Delay Requirement	441

8.9.3	PSCell Release Delay Requirement	442

8.9A	Conditional PSCell Addition Delay	442

8.9A.1	Introduction	442

8.9A.2	Conditional PSCell Addition Delay Requirement	442

8.9A.2.1	Measurement time	443

8.9B	NR-DC: PSCell Addition and Release Delay in Carriers with CCA	443

8.9B.1	Introduction	443

8.9B.2	PSCell Addition Delay Requirement	444

8.9B.3	PSCell Release Delay Requirement	444

8.10	Active TCI state switching delay	445

8.10.3A	MAC-CE based TCI state switch delay in HST FR2 scenarios	446

8.10.4	DCI based TCI state switch delay	446

8.10.5	RRC based TCI state switch delay	447

8.10.6	Active TCI state list update delay	447

8.10A	Active TCI state switching delay with CCA	447

8.10A.1	Introduction	447

8.10A.2	Known conditions for TCI state	448

8.10A.3	MAC-CE based TCI state switch delay	448

8.10A.4	DCI based TCI state switch delay	449

8.10A.5	RRC based TCI state switch delay	449

8.10A.6	Active TCI state list update delay	450

8.10B	Active TCI state switching delay for RedCap	450

8.10B.1	Introduction	450

8.10B.2	Known conditions for TCI state	450

8.10B.3	MAC-CE based TCI state switch delay	451

8.10B.4	DCI based TCI state switch delay	451

8.10B.5	RRC based TCI state switch delay	452

8.10B.6	Active TCI state list update delay	452

8.10C	Active TCI state switching delay for satellite access	453

8.10C.1	Introduction	453

8.10C.2	MAC-CE based TCI state switch delay	453

8.10C.4	DCI based TCI state switch delay	453

8.10C.5	RRC based TCI state switch delay	453

8.10C.6	Active TCI state list update delay	453

8.11	PSCell Change	454

8.11A	PSCell Change in Carriers with CCA	454

8.11B	Conditional PSCell Change	454

8.11B.1	Introduction	454

8.11B.2	Conditoinal PSCell Change delay	455

8.11B.2.1	Measurement time	455

8.11D	Conditional PSCell Change in Carriers with CCA	456

8.11D.1	Introduction	456

8.11D.2	Conditional PSCell Change delay	456

8.11D.2.1	Measurement time	457

8.12	Uplink spatial relation switch delay	457

8.12.1	Introduction	457

8.12.2	Known conditions for spatial relation when associated with DL-RS	457

8.12.3	MAC-CE based spatial relation switch delay	458

8.12.4	DCI based spatial relation switch delay	458

8.12.5	RRC based spatial relation switch delay	459

8.12A	Uplink spatial relation switch delay for RedCap	459

8.12A.1	Introduction	459

8.12A.2	Known conditions for spatial relation when associated with DL-RS	459

8.12A.3	MAC-CE based spatial relation switch delay	460

8.12A.4	DCI based spatial relation switch delay	460

8.12A.5	RRC based spatial relation switch delay	461

8.12C	Uplink spatial relation switch delay for satellite access	461

8.12C.1	Void	462

8.12C.2	Void	462

8.12C.3	Void	462

8.12C.4	Void	462

8.12C.5	Void	462

8.13	UE-specific CBW change	462

8.13.1	Introduction	462

8.13.2	UE-specific CBW change delay	462

8.13A	UE-specific CBW change for RedCap	462

8.13A.1	Introduction	462

8.13A.2	UE-specific CBW change delay	462

8.13C	UE-specific CBW change for satellite access	463

8.13C.1	Introduction	463

8.13C.2	UE-specific CBW change delay	463

8.14	Pathloss reference signal switching delay	463

8.14.1	Introduction	463

8.14.2	Known conditions for pathloss reference signal	464

8.14.3	MAC-CE based pathloss reference signal switch delay	464

8.14C	Pathloss reference signal switching delay for satellite access	464

8.14C.1	Introduction	465

8.14C.2	Known conditions for pathloss reference signal	465

8.14C.3	MAC-CE based pathloss reference signal switch delay	465

8.15	Active downlink TCI state switching delay for unified TCI	466

8.15.1	Introduction	466

8.15.4	DCI based downlink TCI state switch delay	467

8.15.5	Active Downlink TCI state list update delay	468

8.16	Active uplink TCI state switching delay for unified TCI	469

8.16.1	Introduction	469

8.16.4	DCI based uplink TCI state switch delay	471

8.16.5	Active Uplink TCI state list update delay	471

8.17	SCG Activation and Deactivation Delay	473

8.17.1	Introduction	473

8.17.2	SCG Activation Delay Requirement	473

8.17.3	SCG Deactivation Delay Requirement	474

8.18	TRP specific Link Recovery Procedures	474

8.18.1	Introduction	474

8.18.2	Requirements for TRP specific SSB based beam failure detection	475

8.18.2.1	Introduction	475

8.18.2.2	Minimum requirement	475

8.18.2.3	Measurement restriction for SSB based beam failure detection	477

8.18.3	Requirements for CSI-RS based beam failure detection	478

8.18.3.1	Introduction	478

8.18.3.2	Minimum requirement	478

8.18.3.3	Measurement restrictions for CSI-RS beam failure detection	480

8.18.4	Minimum requirement for L1 indication	481

8.18.5	Requirements for SSB based candidate beam detection	481

8.18.5.1	Introduction	481

8.18.5.2	Minimum requirement	481

8.18.5.3	Measurement restriction for SSB based candidate beam detection	483

8.18.6	Requirements for CSI-RS based candidate beam detection	484

8.18.6.1	Introduction	484

8.18.6.2	Minimum requirement	484

8.18.6.3	Measurement restriction for CSI-RS based candidate beam detection	486

8.18.7	Requirements for TRP specific Beam Failure Recovery	487

8.18.7.1	Introduction	487

8.18.7.2	Requirement	487

8.18.8	Scheduling availability of UE during TRP specific beam failure detection	487

8.18.8.1	Scheduling availability of UE performing TRP specific beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	487

8.18.8.2	Scheduling availability of UE performing TRP specific beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	488

8.18.8.3	Scheduling availability of UE performing TRP specific beam failure detection on FR2	488

8.18.8.4	Scheduling availability of UE performing TRP specific beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	489

8.18.9	Scheduling availability of UE during TRP specific candidate beam detection	489

8.18.9.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	489

8.18.9.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	489

8.18.9.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	489

8.18.9.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	490

8.19	Pre-configured measurement gap activation/deactivation delay	490

8.19.1	Introduction	490

8.19.2	Pre-configured measurement gap activation/deactivation upon DCI/timer-based BWP switch	490

8.19.2.1	Activation/deactivation upon DCI/timer-based BWP switch delay on a single CC	490

8.19.3	Pre-configured measurement gap activation/deactivation upon SCell activation/deactivation	490

8.19.4	Pre-configured measurement gap activation/deactivation upon RRC reconfiguration	491

9	Measurement Procedure	491

9.1	General measurement requirement	491

9.1.1	Introduction	491

9.1.2	Measurement gap	491

9.1.2.1	EN-DC: Measurement Gap Sharing	503

9.1.2.1a	SA: Measurement Gap Sharing	504

9.1.2.1b	NE-DC: Measurement Gap Sharing	505

9.1.2.1c	NR-DC: Measurement Gap Sharing	506

9.1.3	UE Measurement capability	507

9.1.3.1	EN-DC: Monitoring of multiple layers using gaps	507

9.1.3.1a	SA: Monitoring of multiple layers using gaps	507

9.1.3.1b	NE-DC: Monitoring of multiple layers using gaps	508

9.1.3.1c	NR-DC: Monitoring of multiple layers using gaps	508

9.1.3.2	EN-DC: Maximum allowed layers for multiple monitoring	509

9.1.3.2a	SA: Maximum allowed layers for multiple monitoring	510

9.1.3.2b	NE-DC: Maximum allowed layers for multiple monitoring	510

9.1.3.2c	NR-DC: Maximum allowed layers for multiple monitoring	511

9.1A.3.2	Void	512

9.1.3A	UE Measurement capability under operation mode with CCA	512

9.1.3A.1	EN-DC: Monitoring of multiple layers using gaps under CCA	512

9.1.3A.1a	SA: Monitoring of multiple layers using gaps under CCA	512

9.1.3A.2	EN-DC: Maximum allowed layers for multiple monitoring under CCA	512

9.1.3A.2a	SA: Maximum allowed layers for multiple monitoring under CCA	513

9.1.3C	UE Measurement capability under operation mode with satellite access	513

9.1.3C.1a	SA: Monitoring of multiple layers using gaps under satellite access	513

9.1.3C.2a	SA: Maximum allowed layers for multiple monitoring for SAN	514

9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria	514

9.1.4.1	Introduction	514

9.1.4.2	Requirements	514

9.1.5	Carrier-specific scaling factor	518

9.1.5.1	Monitoring of multiple layers outside gaps	519

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	521

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	522

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	523

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps	524

9.1.5.2	Monitoring of multiple layers within gaps	525

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	527

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	528

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps	530

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps	532

9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps	534

9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	534

9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	534

9.1.5.3	Monitoring of multiple layers within NCSG	534

9.1.5.3.1	SA mode: carrier-specific scaling factor for measurements performed within NCSG	535

9.1.6	Minimum requirement at transitions	536

9.1.7	Pre-configured measurement gap	536

9.1.7.1	Introduction	536

9.1.7.2	Requirements applicability	536

9.1.7.3	Requirements	537

9.1.7.3.1	Requirements for autonomous activation/deactivation mechanism	537

9.1.7.3.2	Requirements for network-controlled activation/deactivation mechanism	538

9.1.8	Concurrent measurement gaps	538

9.1.8.1	Introduction	538

9.1.8.2	Requirements	538

9.1.8.3	Collision between concurrent measurement gaps	540

9.1.8.4	Measurement gap related requirements of concurrent measurement gaps	540

9.1.9	Network controlled small gap	540

9.1.9.1	Introduction	540

9.1.9.2	Requirements applicability	541

9.1.10	MUSIM gaps	544

9.1.11	UL gap for Tx power management	545

9.1A	General measurement requirement for RedCap	546

9.1A.1	Introduction	546

9.1A.2	Measurement gap	546

9.1A.2.1	SA: Measurement Gap Sharing	550

9.1A.3	UE Measurement capability	551

9.1A.3.1	SA: Monitoring of multiple layers using gaps	551

9.1A.3.2	SA: Maximum allowed layers for multiple monitoring	551

9.1A.4	Capabilities for Support of Event Triggering and Reporting Criteria	551

9.1A.4.1	Introduction	551

9.1A.4.2	Requirements	552

9.1A.5	Carrier-specific scaling factor	552

9.1A.5.1	Monitoring of multiple layers outside gaps	552

9.1A.5.1.1	SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps	553

9.1A.5.2	Monitoring of multiple layers within gaps	553

9.1A.5.2.1	SA mode: carrier-specific scaling factor for SSB measurements performed within gaps	553

9.1A.6	Minimum requirement at transitions	554

9.1C	General measurement requirement	555

9.1C.1	Introduction	555

9.1C.2	Measurement gap	555

9.1C.8	Concurrent measurement gaps for SAN	557

9.1C.8.1	Introduction	557

9.1C.8.2	Requirements	557

9.1C.8.3	Collision between concurrent measurement gaps	558

9.1C.8.4	Measurement gap related requirements of concurrent measurement gaps	558

9.1C.9	Collision between SMTC and measurement gap for SAN	558

9.1C.9.1	Introduction	558

9.1C.9.1	Collision between SMTCs and measurement gap	558

9.2	NR intra-frequency measurements	559

9.2.1	Introduction	559

9.2.2	Requirements applicability	560

9.2.3	Number of cells and number of SSB	560

9.2.3.1	Requirements for FR1	560

9.2.3.2	Requirements for FR2	560

9.2.4	Measurement Reporting Requirements	561

9.2.4.1	Periodic Reporting	561

9.2.4.2	Event-triggered Periodic Reporting	561

9.2.4.3	Event Triggered Reporting	561

9.2.5	Intrafrequency measurements without measurement gaps	562

9.2.5.1	Intrafrequency cell identification	562

9.2.5.2	Measurement period	567

9.2.5.3	Scheduling availability of UE during intra-frequency measurements	570

9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	570

9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	570

9.2.5.3.3	Scheduling availability of UE performing measurements on FR2	571

9.2.5.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	572

9.2.5.4	SFTD Measurements between PCell and PSCell	573

9.2.5.4.1	Introduction	573

9.2.5.4.2	SFTD Measurement delay	573

9.2.5.4.3	SFTD Measurement Reporting Delay	573

9.2.6	Intra-frequency measurements with measurement gaps	574

9.2.6.1	Void	574

9.2.6.2	Intra-frequency cell identification	574

9.2.6.3	Intrafrequency Measurement Period	576

9.2.7	Intra-frequency measurements with NCSG	578

9.2.7.1	Intra-frequency cell identification	578

9.2.7.2	Measurement period	580

9.2.7.3	Scheduling availability during intra-frequency measurement with NCSG	581

9.2A	NR intra-frequency measurements with CCA	581

9.2A.1	Introduction	581

9.2A.2	Requirements applicability	582

9.2A.3	Number of cells and number of SSB	582

9.2A.3.1	Requirements for FR1	582

9.2A.3.2	Requirements for FR2-2	583

9.2A.4	Measurement Reporting Requirements	583

9.2A.5	Intra-frequency measurements without measurement gaps	584

9.2A.5.2	Measurement period	588

9.2A.5.3	Scheduling availability of UE during intra-frequency measurements	590

9.2A.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	591

9.2A.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	591

9.2A.5.3.3	Scheduling availability of UE performing measurements in TDD bands on FR2-2	591

9.2A.6	Intra-frequency measurements with measurement gaps	591

9.2A.6.1	Intra-frequency cell identification	591

9.2A.6.2	Intra-frequency Measurement Period	594

9.2A.7	Intra-frequency RSSI and Channel occupancy measurements	595

9.2A.7.1	Intra-frequency RSSI measurements	595

9.2A.7.2	Intra-frequency Channel occupancy measurements	597

9.2A.7.3	Scheduling restriction during RSSI and Channel Occupancy measurements in FR1	598

9.2A.7.4	Scheduling restriction during RSSI measurements in FR2-2	598

9.2B	NR intra-frequency measurements for RedCap	599

9.2B.1	Introduction	599

9.2B.2	Requirements applicability	599

9.2B.3	Number of cells and number of SSB	600

9.2B.3.1	Requirements for FR1	600

9.2B.3.2	Requirements for FR2	600

9.2B.4	Measurement Reporting Requirements	600

9.2B.4.1	Periodic Reporting	600

9.2B.4.2	Event-triggered Periodic Reporting	600

9.2B.4.3	Event Triggered Reporting	600

9.2B.5	Intra-frequency measurements without measurement gaps for RedCap	601

9.2B.5.1	Intra-frequency cell identification	601

9.2B.5.2	Measurement period	603

9.2B.5.3	Scheduling availability of UE during intra-frequency measurements	604

9.2B.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	604

9.2B.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	605

9.2B.5.3.3	Scheduling availability of UE performing measurements on FR2	605

9.2B.5.3.4	Scheduling availability of UE performing measurements in HD-FDD bands on FR1	606

9.2B.6	Intra-frequency measurements with measurement gaps	606

9.2B.6.1	Intra-frequency cell identification	606

9.2B.6.2	Intra-frequency Measurement Period	608

9.2C	NR intra-frequency measurements for SAN	608

9.2C.1	Introduction	608

9.2C.2	Requirements applicability	609

9.2C.3	Number of cells and number of SSB	609

9.2C.3.1	Requirements for FR1	609

9.2C.4	Measurement Reporting Requirements	610

9.2C.4.1	Periodic Reporting	610

9.2C.4.2	Event-triggered Periodic Reporting	610

9.2C.4.3	Event Triggered Reporting	610

9.2C.5	Intra frequency measurements without measurement gaps	610

9.2C.5.1	Intra frequency cell identification	610

9.2C.5.2	Measurement period	613

9.2C.5.3	Scheduling availability of UE during intra-frequency measurements	613

9.2C.5.3.1	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	613

9.2C.5.3.2	Scheduling availability of UE performing measurements on a neighbor cell served by a different satellite in LEO	613

9.2C.6	Intra-frequency measurements with measurement gaps	614

9.2C.6.1	Intra-frequency cell identification	614

9.2C.6.3	Intrafrequency Measurement Period	615

9.3	NR inter-frequency measurements	615

9.3.1	Introduction	615

9.3.2	Requirements applicability	616

9.3.2.1	Void	617

9.3.2.2	Void	617

9.3.3	Number of cells and number of SSB	617

9.3.3.1	Requirements for FR1	617

9.3.3.2	Requirements for FR2	617

9.3.4	Inter-frequency measurement with measurement gaps	617

9.3.4.1	Void	620

9.3.4.2	Void	620

9.3.5	Inter-frequency measurements	620

9.3.5.1	Void	621

9.3.5.2	Void	621

9.3.5.3	Void	621

9.3.6	Inter-frequency measurements reporting requirements	621

9.3.6.1	Periodic Reporting	621

9.3.6.2	Event-triggered Periodic Reporting	621

9.3.6.3	Event-triggered Reporting	621

9.3.7	Void	622

9.3.8	Inter-frequency SFTD measurement requirements	622

9.3.8.1	Introduction	622

9.3.8.2	SFTD Measurement delay	622

9.3.8.3	SFTD Measurement reporting delay	623

9.3.9	Inter frequency measurements without measurement gaps	623

9.3.9.1	Inter frequency Cell identification	623

9.3.9.2	Measurement period	626

9.3.9.3	Scheduling availability of UE during inter-frequency measurements	626

9.3.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	627

9.3.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	627

9.3.9.3.3	Scheduling availability of UE performing measurements on FR2	627

9.3.9.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	628

9.3.10	Inter-frequency measurement with NCSG	628

9.3.10.1	Inter-frequency cell identification	628

9.3.10.2	Measurement period	629

9.3.10.3	Scheduling availability during inter-frequency measurement with NCSG	630

9.3.10.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	630

9.3.10.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	631

9.3.10.3.3	Scheduling availability of UE performing measurements on FR2	631

9.3.10.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	634

9.3A	NR inter-frequency measurements in carrier frequencies with CCA	634

9.3A.1	Introduction	634

9.3A.2	Requirements applicability	634

9.3A.3	Number of cells and number of SSB	635

9.3A.3.1	Requirements for FR1	635

9.3A.3.2	Requirements for FR2-2	635

9.3A.4	Inter-frequency cell identification	635

9.3A.5	Inter-frequency measurements	637

9.3A.6	NR Inter-frequency measurements reporting requirements	639

9.3A.6.1	Periodic Reporting	639

9.3A.6.2	Event-triggered Periodic Reporting	639

9.3A.6.3	Event-triggered Reporting	639

9.3A.8	Inter-frequency RSSI measurements	639

9.3A.9	Inter-frequency channel occupancy measurements	640

9.3B	NR inter-frequency measurements for RedCap	641

9.3B.1	Introduction	641

9.3B.2	Requirements applicability	641

9.3B.3	Number of cells and number of SSB	642

9.3B.3.1	Requirements for FR1	642

9.3B.3.2	Requirements for FR2	642

9.3B.4	Inter-frequency measurement with measurement gaps	642

9.3B.5	Inter-frequency measurements	644

9.3B.6	Inter-frequency measurements reporting requirements	644

9.3B.6.1	Periodic Reporting	644

9.3B.6.2	Event-triggered Periodic Reporting	644

9.3B.6.3	Event-triggered Reporting	645

9.3B.7	Inter frequency measurements without measurement gaps	645

9.3B.7.1	Inter frequency Cell identification	645

9.3B.7.2	Measurement period	648

9.3B.7.3	Scheduling availability of UE during inter-frequency measurements	648

9.3B.7.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	649

9.3B.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	649

9.3B.7.3.3	Scheduling availability of UE performing measurements on FR2	649

9.3B.7.3.4	Scheduling availability of UE performing measurements in HD-FDD bands on FR1	650

9.3C	NR inter-frequency measurements for SAN	650

9.3C.1	Introduction	650

9.3C.2	Requirements applicability	650

9.3C.3	Number of cells and number of SSB	651

9.3C.3.1	Requirements for FR1	651

9.3C.4	Inter-frequency measurement with measurement gaps	651

9.3C.5	Inter-frequency measurements	652

9.3C.6	Inter-frequency measurements reporting requirements	652

9.3C.6.1	Periodic Reporting	652

9.3C.6.2	Event-triggered Periodic Reporting	653

9.3C.6.3	Event-triggered Reporting	653

9.3C.7	Inter frequency measurements without measurement gaps	653

9.3C.7.1	Inter frequency Cell identification	653

9.3C.7.2	Measurement period	655

9.3C.7.3	Scheduling availability of UE during inter-frequency measurements	655

9.3C.7.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	656

9.3C.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	656

9.4	Inter-RAT measurements	656

9.4.1	Introduction	656

9.4.2	NR − E-UTRAN FDD measurements	659

9.4.2.1	Introduction	659

9.4.2.2	Requirements when no DRX is used	659

9.4.2.3	Requirements when DRX is used	660

9.4.2.4	Measurement reporting requirements	662

9.4.2.4.1	Periodic Reporting	662

9.4.2.4.2	Event-Triggered Periodic Reporting	662

9.4.2.4.3	Event-Triggered Reporting	662

9.4.3	NR − E-UTRAN TDD measurements	662

9.4.3.1	Introduction	662

9.4.3.2	Requirements when no DRX is used	663

9.4.3.3	Requirements when DRX is used	664

9.4.3.4	Measurement reporting requirements	666

9.4.3.4.1	Periodic Reporting	666

9.4.3.4.2	Event-Triggered Periodic Reporting	666

9.4.3.4.3	Event-Triggered Reporting	666

9.4.3.5	Scheduling Availability During NR − E-UTRAN TDD measurements with NCSG	667

9.4.4	Inter-RAT RSTD measurements	667

9.4.4.1	NR − E-UTRAN FDD RSTD measurements	667

9.4.4.1.1	Introduction	667

9.4.4.1.2	Requirements	667

9.4.4.2	NR − E-UTRAN TDD RSTD measurements	670

9.4.4.2.1	Introduction	670

9.4.4.2.2	Requirements	671

9.4.5	Inter-RAT E-CID measurements	674

9.4.5.1	NR−E-UTRAN FDD E-CID RSRP and RSRQ measurements	674

9.4.5.1.1	Introduction	674

9.4.5.1.2	Requirements	674

9.4.5.1.3	Measurement Reporting Delay	675

9.4.5.2	NR−E-UTRAN TDD E-CID RSRP and RSRQ measurements	675

9.4.5.2.1	Introduction	675

9.4.5.2.2	Requirements	675

9.4.5.2.3	Measurement Reporting Delay	675

9.4.6	NR − UTRAN FDD measurements	675

9.4.6.1	Introduction	675

9.4.6.2	Requirements when no DRX is used	675

9.4.6.3	Requirements when DRX is used	677

9.4.7	NR – E-UTRAN measurements with autonomous gaps	679

9.4.7.1	CGI identification of an E-UTRA cell with autonomous gaps	679

9.4A	Inter-RAT measurements for RedCap	679

9.4A.1	Introduction	679

9.4A.2	NR − E-UTRAN FDD measurements	680

9.4A.2.1	Introduction	680

9.4A.2.2	Requirements when no DRX is used	681

9.4A.2.3	Requirements when DRX is used	682

9.4A.2.4	Measurement reporting requirements	683

9.4A.2.4.1	Periodic Reporting	683

9.4A.2.4.2	Event-Triggered Periodic Reporting	683

9.4A.2.4.3	Event-Triggered Reporting	683

9.4A.3	NR − E-UTRAN TDD measurements	684

9.4A.3.1	Introduction	684

9.4A.3.2	Requirements when no DRX is used	684

9.4A.3.3	Requirements when DRX is used	686

9.4A.3.4	Measurement reporting requirements	687

9.4A.3.4.1	Periodic Reporting	687

9.4A.3.4.2	Event-Triggered Periodic Reporting	687

9.4A.3.4.3	Event-Triggered Reporting	687

9.4A.4	NR – E-UTRAN measurements with autonomous gaps	688

9.4A.4.1	CGI identification of an E-UTRA cell with autonomous gaps	688

9.4A.4.2	CGI reporting delay	688

9.4A.4.3	CGI reporting scheduling restriction	689

9.5	L1-RSRP measurements for Reporting	689

9.5.1	Introduction	689

9.5.2	Requirements applicability	689

9.5.3	Measurement Reporting Requirements	690

9.5.3.1	Periodic Reporting	690

9.5.3.2	Semi-Persistent Reporting	690

9.5.3.3	Aperiodic Reporting	691

9.5.4	L1-RSRP measurement requirements	691

9.5.4.1	SSB based L1-RSRP Reporting	691

9.5.4.2	CSI-RS based L1-RSRP Reporting	694

9.5.4A	Void	697

9.5.4A.1	Void	697

9.5.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	698

9.5.5.1	Measurement restriction for SSB based L1-RSRP	698

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP	698

9.5.6	Scheduling availability of UE during L1-RSRP measurement	699

9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	699

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	699

9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	699

9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	701

9.5A	L1-RSRP measurements for Reporting under CCA	701

9.5A.1	Introduction	701

9.5A.2	Requirements applicability	701

9.5A.3	Measurement Reporting Requirements	702

9.5A.3.1	Periodic Reporting	702

9.5A.3.2	Semi-Persistent Reporting	702

9.5A.3.3	Aperiodic Reporting	702

9.5A.4	L1-RSRP measurement requirements	703

9.5A.4.1	SSB based L1-RSRP Reporting	703

9.5A.5	Measurement restriction for L1-RSRP measurement	705

9.5A.5.1	Measurement restriction for SSB based L1-RSRP	705

9.5A.6	Scheduling availability of UE during L1-RSRP measurement	706

9.5A.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	706

9.5A.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	706

9.5A.6.3	Scheduling availability of UE performing L1-RSRP measurement in case of FR1-FR2 inter-band CA	706

9.5A.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2-2	706

9.5A.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	707

9.5B	L1-RSRP measurements for Reporting for RedCap	707

9.5B.1	Introduction	707

9.5B.2	Requirements applicability	707

9.5B.3	Measurement Reporting Requirements	708

9.5B.3.1	Periodic Reporting	708

9.5B.3.2	Semi-Persistent Reporting	708

9.5B.3.3	Aperiodic Reporting	709

9.5B.4	L1-RSRP measurement requirements	709

9.5B.4.1	SSB based L1-RSRP Reporting	709

9.5B.4.2	CSI-RS based L1-RSRP Reporting	711

9.5B.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	714

9.5B.5.1	Measurement restriction for SSB based L1-RSRP	714

9.5B.5.2	Measurement restriction for CSI-RS based L1-RSRP	714

9.5B.6	Scheduling availability of UE during L1-RSRP measurement	714

9.5B.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	715

9.5B.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	715

9.5B.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	715

9.5C	L1-RSRP measurements for Reporting for satellite access	716

9.5C.1	Introduction	716

9.5C.3	Measurement Reporting Requirements	716

9.5C.3.1	Periodic Reporting	717

9.5C.3.2	Semi-Persistent Reporting	717

9.5C.3.3	Aperiodic Reporting	717

9.5C.4	L1-RSRP measurement requirements	717

9.5C.4.1	SSB based L1-RSRP Reporting	717

9.5C.5	Measurement restriction for L1-RSRP measurement	719

9.5C.5.1	Measurement restriction for SSB based L1-RSRP	719

9.5C.5.2	Measurement restriction for CSI-RS based L1-RSRP	719

9.5C.6	Scheduling availability of UE during L1-RSRP measurement	720

9.5C.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	720

9.5C.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	720

9.6	NE-DC: Measurements	720

9.6.1	Introduction	720

9.6.2	SFTD Measurements	720

9.6.2.1	Introduction	720

9.6.2.2	SFTD Measurement requirements	720

9.7	Cross Link Interference measurements	721

9.7.1	Introduction	721

9.7.2	SRS-RSRP measurements	721

9.7.2.1	Introduction	721

9.7.2.2	Requirements applicability	722

9.7.2.3	Measurement Reporting Requirements	722

9.7.2.3.1	Periodic Reporting	722

9.7.2.3.2	Event-triggered Periodic Reporting	722

9.7.2.3.3	Event Triggered Reporting	722

9.7.2.4	Measurement capability	722

9.7.2.5	SRS-RSRP measurement period	723

9.7.3	CLI-RSSI measurements	723

9.7.3.1	Introduction	723

9.7.3.2	Requirements applicability	723

9.7.3.3	Measurement Reporting Requirements	723

9.7.3.3.1	Periodic Reporting	723

9.7.3.3.2	Event-triggered Periodic Reporting	723

9.7.3.3.3	Event Triggered Reporting	724

9.7.3.4	Measurement capability	724

9.7.3.5	CLI-RSSI measurement period	724

9.7.4	Scheduling availability of UE during CLI measurements	724

9.7.4.1	Scheduling availability of UE performing measurement on FR1	724

9.7.4.2	Scheduling availability of UE performing measurement on FR2	725

9.8	L1-SINR measurements for Reporting	725

9.8.1	Introduction	725

9.8.2	Requirements applicability	726

9.8.3	Measurement Reporting Requirements	726

9.8.3.1	Periodic Reporting	726

9.8.3.2	Semi-Persistent Reporting	727

9.8.4	L1-SINR measurement requirements	727

9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	727

9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	732

9.8.5	Measurement restriction for L1-SINR measurement	733

9.8.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	734

9.8.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	734

9.8.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	735

9.8.6	Scheduling availability of UE during L1-SINR measurement	735

9.8.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	735

9.8.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	735

9.8.6.4	Scheduling availability of UE performing L1-SINR measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	736

9.9	 NR measurements for positioning	737

9.9.1	Introduction	737

9.9.1.1	General Aspects of Gap-based Measurement	737

9.9.1.2	General Aspects of Gapless Measurement	737

9.9.1.3	Scheduling Availability of UE during PRS Measurement without Measurement Gaps	738

9.9.2	RSTD measurements	739

9.9.2.1	Introduction	739

9.9.2.2	Requirements Applicability	739

9.9.2.3	Measurement Capability	739

9.9.2.4	Measurement Reporting Requirements	739

9.9.2.4.1	Void	740

9.9.2.4.2	Void	740

9.9.2.4.3	Void	740

9.9.2.5	Measurements Period Requirements	740

9.9.2.6	Void	743

9.9.2.7	Measurements Period Requirements without Measurement Gaps	743

9.9.2.8	Void	746

9.9.3	PRS-RSRP measurements	746

9.9.3.1	Introduction	746

9.9.3.2	Requirements applicability	746

9.9.3.3	Measurement Capability	746

9.9.3.4	Measurement Reporting Requirements	746

9.9.3.5	Measurement Period Requirements	746

9.9.3.6	Measurement Period Requirements without Measurement Gaps	749

9.9.3.7	Void	751

9.9.4	UE Rx-Tx time difference measurements	751

9.9.4.1 Introduction	751

9.9.4.2 Requirements Applicability	751

9.9.4.3 Measurement Capability	752

9.9.4.4 Measurement Reporting Requirements	752

9.9.4.5	Measurement Period Requirements	752

9.9.4.6	Measurement Period Requirements without Measurement Gaps	755

9.9.4.7	Void	759

9.9.5	E-CID measurements	759

9.9.5.1	Introduction	759

9.9.5.2	Measurement Requirements	759

9.9.5.2.1	Intra-frequency Measurement Requirements	759

9.9.5.2.2	Inter-frequency Measurement Requirements	759

9.9.5.2.3	Measurement Reporting Delay	760

9.9.6	PRS-RSRPP measurements	760

9.9.6.1	Introduction	760

9.9.6.2	Requirements applicability	760

9.9.6.3	Measurement capability	760

9.9.6.4	Measurement reporting requirements	760

9.9.6.5	Measurement period requirements	760

9.9.6.6	Measurement Period Requirements without Measurement Gaps	761

9.9.6.7	Void	761

9.10	CSI-RS based L3 measurements	761

9.10.1	Introduction	761

9.10.2	CSI-RS based intra-frequency measurements	761

9.10.2.1	Introduction	761

9.10.2.2	Requirements applicability	762

9.10.2.3	Number of cells and number of CSI-RS	763

9.10.2.3.1	Requirements for FR1	763

9.10.2.3.2	Requirements for FR2	763

9.10.2.4	Measurement Reporting Requirements	763

9.10.2.4.1	Periodic Reporting	763

9.10.2.4.2	Event-triggered Periodic Reporting	763

9.10.2.4.3	Event Triggered Reporting	764

9.10.2.5	Intra-frequency measurements without measurement gaps	764

9.10.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	766

9.10.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	766

9.10.2.6.2	Scheduling availability of UE performing CSI-RS based measurements in FR2	766

9.10.3	CSI-RS based Inter-frequency measurements	766

9.10.3.1	Introduction	766

9.10.3.2	Requirements applicability	767

9.10.3.3	Number of cells and number of CSI-RS resources	767

9.10.3.3.1	Requirements for FR1	767

9.10.3.3.2	Requirements for FR2	768

9.10.3.4	Measurements reporting requirements	768

9.10.3.4.1	Periodic Reporting	768

9.10.3.4.2	Event-triggered Periodic Reporting	768

9.10.3.4.3	Event-triggered Reporting	768

9.10.3.5	Inter frequency measurements with measurement gaps	768

9.11	NR measurements with autonomous gaps	770

9.11.1	Introduction	770

9.11.2	CGI identification of an NR cell with autonomous gaps	770

9.11.3	CGI reporting delay	771

9.11A	NR measurements with autonomous gaps for RedCap	772

9.11A.1	Introduction	772

9.11A.2	CGI identification of an NR cell with autonomous gaps	772

9.11A.3	CGI reporting delay	773

9.11A.4	CGI reporting scheduling restriction	773

9.12	Measurement for Propagation Delay Compensation	774

9.12.1	Introduction	774

9.12.2	Requirements Applicability	774

9.12.3	Measurement Capability	774

9.12.4	Measurement period requirements	774

9.12.4.1	PRS Measurement Period	774

9.12.4.2	TRS Measurement Period	775

9.12.5	Measurement Reporting Requirements	776

9.12.6	Scheduling availability during measurement for Propagation Delay Compensation	776

9.12.7	Measurement restriction for measurement for Propagation Delay Compensation	776

9.13	L1-RSRP measurements for a cell with different PCI from serving cell	776

9.13.1	Introduction	776

9.13.2	Requirements Applicability	777

9.13.3	Measurement Reporting Requirements	777

9.13.3.1	Periodic Reporting	778

9.13.3.2	Semi-Persistent Reporting	778

9.13.3.3	Aperiodic Reporting	778

9.13.4	L1-RSRP measurement requirements	778

9.13.4.1	Inter-cell SSB based L1-RSRP Reporting	778

9.13.5	Measurement restriction for L1-RSRP measurement	780

9.13.5.1	Measurement restriction for SSB based L1-RSRP	780

9.13.6	Scheduling availability of UE during L1-RSRP measurement	781

9.13.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	781

9.13.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	781

9.13.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	781

9.13.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	782

9.13.6.5	Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1	782

10	Measurement Performance requirements	782

10.1	NR measurements	782

10.1.1	Introduction	782

10.1.2	Intra-frequency RSRP accuracy requirements for FR1	783

10.1.2.1	Intra-frequency SS-RSRP accuracy requirements	783

10.1.2.1.1	Absolute SS-RSRP Accuracy	783

10.1.2.1.2	Relative SS-RSRP Accuracy	784

10.1.2.2	Void	785

10.1.2.3	Intra-frequency CSI-RSRP accuracy requirements	785

10.1.2.3.1	Absolute CSI-RSRP Accuracy	785

10.1.2.3.2	Relative CSI-RSRP Accuracy	786

10.1.2B	Intra-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	787

10.1.2B.1	Intra-frequency SS-RSRP accuracy requirements	787

10.1.2B.1.1	Absolute SS-RSRP Accuracy	787

10.1.2C	Intra-frequency RSRP accuracy requirements for FR1 SAN	788

10.1.2C.1	Intra-frequency SS-RSRP accuracy requirements	788

10.1.2C.1.1	Absolute SS-RSRP Accuracy	788

10.1.2C.1.2	Relative SS-RSRP Accuracy	789

10.1.3	Intra-frequency RSRP accuracy requirements for FR2	789

10.1.3.1	Intra-frequency SS-RSRP accuracy requirements	789

10.1.3.1.1	Absolute SS-RSRP Accuracy	789

10.1.3.1.2	Relative SS-RSRP Accuracy	790

10.1.3.2	Void	791

10.1.3.3	Intra-frequency CSI-RSRP accuracy requirements	791

10.1.3.3.1	Absolute CSI-RSRP Accuracy	791

10.1.3.3.2	Relative CSI-RSRP Accuracy	792

10.1.3B	Intra-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	793

10.1.3B.1	Intra-frequency SS-RSRP accuracy requirements	793

10.1.3B.1.1	Absolute SS-RSRP Accuracy	793

10.1.4	Inter-frequency RSRP accuracy requirements for FR1	794

10.1.4.1	Inter-frequency SS-RSRP accuracy requirements	794

10.1.4.1.1	Absolute Accuracy of SS-RSRP in FR1	794

10.1.4.1.2	Relative Accuracy of SS-RSRP in FR1	795

10.1.4.2	Void	796

10.1.4.3	Inter-frequency CSI-RSRP accuracy requirements	796

10.1.4.3.1	Absolute Accuracy of CSI-RSRP in FR1	796

10.1.4.3.2	Relative Accuracy of CS-RSRP in FR1	797

10.1.4B	Inter-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	798

10.1.4B.1	Inter-frequency SS-RSRP accuracy requirements	798

10.1.4B.1.1	Absolute Accuracy of SS-RSRP in FR1	798

10.1.4C	Inter-frequency RSRP accuracy requirements for FR1 SAN	799

10.1.4C.1	Inter-frequency SS-RSRP accuracy requirements	799

10.1.4C.1.1	Absolute Accuracy of SS-RSRP in FR1	799

10.1.4C.1.2	Relative Accuracy of SS-RSRP in FR1	800

10.1.5	Inter-frequency RSRP accuracy requirements for FR2	800

10.1.5.1	Inter-frequency SS-RSRP accuracy requirements	800

10.1.5.1.1	Absolute SS-RSRP Accuracy	800

10.1.5.1.2	Relative SS-RSRP Accuracy	801

10.1.5.2	Void	802

10.1.5.3	Inter-frequency CSI-RSRP accuracy requirements	802

10.1.5.3.1	Absolute CSI-RSRP Accuracy	802

10.1.5.3.2	Relative CSI-RSRP Accuracy	803

10.1.5B	Inter-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	804

10.1.5B.1	Inter-frequency SS-RSRP accuracy requirements	804

10.1.5B.1.1	Absolute SS-RSRP Accuracy	804

10.1.6	RSRP Measurement Report Mapping	805

10.1.7	Intra-frequency RSRQ accuracy requirements for FR1	807

10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	807

10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1	807

10.1.7.2	Intra-frequency CSI-RSRQ accuracy requirements	808

10.1.7.2.1	Absolute CSI-RSRQ Accuracy	808

10.1.7B	Intra-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	809

10.1.7B.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	809

10.1.7B.1.1	Absolute SS-RSRQ Accuracy in FR1	809

10.1.7C	Intra-frequency RSRQ accuracy requirements for FR1 SAN	810

10.1.7C.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	810

10.1.7C.1.1	Absolute SS-RSRQ Accuracy in FR1	810

10.1.8	Intra-frequency RSRQ accuracy requirements for FR2	811

10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	811

10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2	811

10.1.8.2	Intra-frequency CSI-RSRQ accuracy requirements	812

10.1.8.2.1	Absolute CSI-RSRQ Accuracy	812

10.1.8B	Intra-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	812

10.1.8B.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	812

10.1.8B.1.1	Absolute SS-RSRQ Accuracy in FR2	813

10.1.9	Inter-frequency RSRQ accuracy requirements for FR1	813

10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	813

10.1.9.1.1	Absolute Accuracy of SS-RSRQ in FR1	813

10.1.9.1.2	Relative Accuracy of SS-RSRQ in FR1	814

10.1.9.2	Inter-frequency CSI-RSRQ accuracy requirements	815

10.1.9.2.1	Absolute CSI-RSRQ Accuracy	815

10.1.9.2.2	Relative CSI-RSRQ Accuracy	816

10.1.9B	Inter-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	817

10.1.9B.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	817

10.1.9B.1.1	Absolute Accuracy of SS-RSRQ in FR1	817

10.1.9C	Inter-frequency RSRQ accuracy requirements for FR1 SAN	818

10.1.9C.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	818

10.1.9C.1.1	Absolute Accuracy of SS-RSRQ in FR1	818

10.1.9C.1.2	Relative Accuracy of SS-RSRQ in FR1	819

10.1.10	Inter-frequency RSRQ accuracy requirements for FR2	820

10.1.10.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	820

10.1.10.1.1	Absolute Accuracy of SS-RSRQ in FR2	820

10.1.10.1.2	Relative Accuracy of SS-RSRQ in FR2	820

10.1.10.2	Inter-frequency CSI-RSRQ accuracy requirements	821

10.1.10.2.1	Absolute CSI-RSRQ Accuracy	821

10.1.10.2.2	Relative CSI-RSRQ Accuracy	822

10.1.10B	 Inter-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	823

10.1.10B.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	823

10.1.10B.1.1	Absolute Accuracy of SS-RSRQ in FR2	823

10.1.11	RSRQ report mapping	824

10.1.12	Intra-frequency SINR accuracy requirements for FR1	824

10.1.12.1	Intra-frequency SS-SINR accuracy requirements in FR1	824

10.1.12.1.1	Absolute SS-SINR Accuracy in FR1	824

10.1.12.2	Intra-frequency CSI-SINR accuracy requirements in FR1	825

10.1.12.2.1	Absolute CSI-SINR Accuracy in FR1	825

10.1.12C	Intra-frequency SINR accuracy requirements for FR1 SAN	826

10.1.12C.1	Intra-frequency SS-SINR accuracy requirements in FR1	826

10.1.12C.1.1	Absolute SS-SINR Accuracy in FR1	826

10.1.13	Intra-frequency SINR accuracy requirements for FR2	827

10.1.13.1	Intra-frequency SS-SINR accuracy requirements in FR2	827

10.1.13.1.1	Absolute SS-SINR Accuracy in FR2	827

10.1.13.2	Intra-frequency CSI-SINR accuracy requirements in FR2	828

10.1.13.2.1	Absolute CSI-SINR Accuracy in FR2	828

10.1.14	Inter-frequency SINR accuracy requirements for FR1	828

10.1.14.1	Inter-frequency SS-SINR accuracy requirements in FR1	828

10.1.14.1.1	Aboslute Accuracy of SS-SINR in FR1	828

10.1.14.1.2	Relative Accuracy of SS-SINR in FR1	829

10.1.14.2	Inter-frequency CSI-SINR accuracy requirements in FR1	830

10.1.14.2.1	Aboslute Accuracy of CSI-SINR in FR1	830

10.1.14.2.2	Relative Accuracy of CSI-SINR in FR1	831

10.1.14C	Inter-frequency SINR accuracy requirements for FR1 SAN	832

10.1.14C.1	Inter-frequency SS-SINR accuracy requirements in FR1	832

10.1.14C.1.1	Aboslute Accuracy of SS-SINR in FR1	832

10.1.14C.1.2	Relative Accuracy of SS-SINR in FR1	833

10.1.15	Inter-frequency SINR accuracy requirements for FR2	834

10.1.15.1	Inter-frequency SS-SINR accuracy requirements in FR2	834

10.1.15.1.1	Aboslute Accuracy of SS-SINR in FR2	834

10.1.15.1.2	Relative Accuracy of SS-SINR in FR2	834

10.1.15.2	Inter-frequency CSI-SINR accuracy requirements in FR2	835

10.1.15.2.1	Aboslute Accuracy of CSI-SINR in FR2	835

10.1.15.2.2	Relative Accuracy of CSI-SINR in FR2	836

10.1.16	SINR report mapping	837

10.1.16.1	SS-SINR and CSI-SINR measurement report mapping	837

10.1.17	Power Headroom	838

10.1.17.1	Power Headroom Report	838

10.1.17.1.1	Power Headroom Report Mapping	838

10.1.18	PCMAX,c,f	839

10.1.18.1	Report Mapping	839

10.1.19	L1-RSRP accuracy requirements for FR1	839

10.1.19.1	SSB based L1-RSRP accuracy requirements	839

10.1.19.1.1	Absolute Accuracy	839

10.1.19.1.2	Relative Accuracy	840

10.1.19.2	CSI-RS based L1-RSRP accuracy requirements	841

10.1.19.2.1	Absolute Accuracy	841

10.1.19.2.2	Relative Accuracy	842

10.1.19C	L1-RSRP accuracy requirements for FR1 SAN	843

10.1.19C.1	SSB based L1-RSRP accuracy requirements	843

10.1.19C.1.1	Absolute Accuracy	843

10.1.19C.1.2	Relative Accuracy	844

10.1.20	L1-RSRP accuracy requirements for FR2	844

10.1.20.1	SSB based L1-RSRP accuracy requirements	844

10.1.20.1.1	Absolute Accuracy	844

10.1.20.1.2	Relative Accuracy	845

10.1.20.2	CSI-RS based L1-RSRP accuracy requirements	846

10.1.20.2.1	Absolute Accuracy	846

10.1.20.2.2	Relative Accuracy	847

10.1.21	SFTD accuracy requirements	848

10.1.21.1	SFTD acuracy requirements for NE-DC	848

10.1.21.2	SFTD acuracy requirements for NR-DC	850

10.1.21.3	Inter frequency SFTD acuracy requirements	851

10.1.22	CLI measurement accuracy requirements	852

10.1.22.1	SRS-RSRP	852

10.1.22.1.1	SRS-RSRP Accuracy	852

10.1.22.1.2	SRS-RSRP report mapping	854

10.1.22.2	CLI-RSSI	854

10.1.22.2.1	CLI-RSSI Accuracy	854

10.1.22.2.2	CLI-RSSI report mapping	855

10.1.23	RSTD Measurements	856

10.1.23.1	Introduction	856

10.1.23.2	Measurement Accuracy Requirements	856

10.1.23.3	Report mapping	862

10.1.23.3.1	Absolute DL RSTD Measurement Reporting	862

10.1.23.3.2	Differential Reporting for DL RSTD Measurement	864

10.1.23.3.3	Additional Path Report Mapping for DL RSTD	866

10.1.24	PRS-RSRP Measurements	868

10.1.24.1	Introduction	868

10.1.24.2	Measurement Accuracy Requirements	868

10.1.24.2.1	Absolute PRS RSRP accuracy	868

10.1.24.2.2	Relative PRS RSRP accuracy	872

10.1.24.3	Report mapping	876

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	876

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	877

10.1.25	UE Rx-Tx Time Difference Measurements	879

10.1.25.1	Introduction	879

10.1.25.2	Measurement Accuracy Requirements	879

10.1.25.3	Report mapping	889

10.1.25.3.1	Absolute UE Rx-Tx Measurement Report Mapping	889

10.1.25.3.2	Differential UE Rx-Tx Measurement Report Mapping	891

10.1.25.3.3	Additional Path Report Mapping for UE Rx-Tx Time Difference	892

10.1.26	FR2 P-MPR report	894

10.1.26.1	Report mapping	894

10.1.27	L1-SINR accuracy requirements for FR1	894

10.1.27.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	894

10.1.27.1.1	Absolute Accuracy	894

10.1.27.1.2	Relative Accuracy	895

10.1.27.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	896

10.1.27.2.1	Absolute Accuracy	896

10.1.27.2.2	Relative Accuracy	897

10.1.27.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	899

10.1.27.3.1	Absolute Accuracy	899

10.1.27.3.2	Relative Accuracy	901

10.1.28	L1-SINR accuracy requirements for FR2	902

10.1.29	Intra-frequency RSRQ accuracy requirements under CCA	909

10.1.29.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	909

10.1.29.1.1	Absolute SS-RSRQ Accuracy	909

10.1.30	Inter-frequency RSRQ accuracy requirements under CCA	909

10.1.30.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	909

10.1.30.1.1	Aboslute Accuracy of SS-RSRQ	909

10.1.30.1.2	Relative Accuracy of SS-RSRQ	910

10.1.31	Intra-frequency SINR accuracy requirements under CCA	911

10.1.31.1	Intra-frequency SS-SINR accuracy requirements in FR1	911

10.1.31.1.1	Absolute SS-SINR Accuracy	911

10.1.32	Inter-frequency SINR accuracy requirements under CCA	911

10.1.32.1	Inter-frequency SS-SINR accuracy requirements in FR1	911

10.1.32.1.1	Aboslute Accuracy of SS-SINR	911

10.1.32.1.2	Relative Accuracy of SS-SINR	912

10.1.33	L1-RSRP accuracy requirements under CCA	913

10.1.33.1	SSB based L1-RSRP accuracy requirements in FR1	913

10.1.33.1.1	Absolute Accuracy	913

10.1.33.1.2	Relative Accuracy	913

10.1.34	RSSI measurements under CCA	914

10.1.34.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	914

10.1.34.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	914

10.1.34.3	RSSI measurement report mapping	914

10.1.35	Channel occupancy measurements under CCA	915

10.1.35.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	915

10.1.35.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	915

10.1.36	Intra-frequency RSRP accuracy requirements under CCA	915

10.1.36.1	Intra-frequency SS-RSRP accuracy requirements in FR1	915

10.1.36.1.1	Absolute SS-RSRP Accuracy	915

10.1.36.1.2	Relative SS-RSRP Accuracy	916

10.1.37	Inter-frequency RSRP accuracy requirements under CCA	917

10.1.37.1	Inter-frequency SS-RSRP accuracy requirements in FR1	917

10.1.37.1.1	Absolute Accuracy of SS-RSRP	917

10.1.37.1.2	Relative Accuracy of SS-RSRP	917

10.1.38	PRS-RSRPP Measurements	918

10.1.38.1	Introduction	918

10.1.38.2	Measurement Accuracy Requirements	918

10.1.38.2.1	Absolute PRS RSRPP accuracy	918

10.1.38.3	Report mapping	922

10.1.38.3.1	Absolute PRS-RSRPP Measurement Report Mapping	922

10.1.38.3.2	Differential Report Mapping for PRS-RSRPP Measurement	923

10.1.39	UE Rx-Tx time difference measurements for RTT-based PDC	924

10.1.39.1	 Introduction	924

10.1.39.2	 Measurement Accuracy Requirements for PRS	924

10.1.39.3	 Measurement Accuracy Requirements for TRS	928

10.1.40	Measurement Accuracy Requirements for Propagation Delay Compensation	931

10.1.40.1	Measurement Accuracy Requirements for PRS	931

10.1.40.2	Measurement Accuracy Requirements for TRS	934

10.1A	NR measurements for RedCap	937

10.1A.1	Introduction	937

10.1A.2	Intra-frequency RSRP accuracy requirements for FR1	937

10.1A.2.1	Intra-frequency SS-RSRP accuracy requirements	937

10.1A.2.1.1	Absolute SS-RSRP Accuracy	937

10.1A.2.1.2	Relative SS-RSRP Accuracy	938

10.1A.3	Intra-frequency RSRP accuracy requirements for FR2	939

10.1A.3.1	Intra-frequency SS-RSRP accuracy requirements	939

10.1A.3.1.1	Absolute SS-RSRP Accuracy	939

10.1A.3.1.2	Relative SS-RSRP Accuracy	939

10.1A.4	Inter-frequency RSRP accuracy requirements for FR1	939

10.1A.4.1	Inter-frequency SS-RSRP accuracy requirements	939

10.1A.4.1.1	Absolute Accuracy of SS-RSRP in FR1	939

10.1A.4.1.2	Relative Accuracy of SS-RSRP in FR1	940

10.1A.5	Inter-frequency RSRP accuracy requirements for FR2	941

10.1A.5.1	Inter-frequency SS-RSRP accuracy requirements	941

10.1A.5.1.1	Absolute SS-RSRP Accuracy	941

10.1A.5.1.2	Relative SS-RSRP Accuracy	941

10.1A.6	Intra-frequency RSRQ accuracy requirements for FR1	941

10.1A.6.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	941

10.1A.6.1.1	Absolute SS-RSRQ Accuracy in FR1	941

10.1A.7	Intra-frequency RSRQ accuracy requirements for FR2	942

10.1A.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	942

10.1A.7.1.1	Absolute SS-RSRQ Accuracy in FR2	942

10.1A.8	Inter-frequency RSRQ accuracy requirements for FR1	942

10.1A.8.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	942

10.1A.8.1.1	Absolute Accuracy of SS-RSRQ in FR1	942

10.1A.8.1.2	Relative Accuracy of SS-RSRQ in FR1	943

10.1A.9	Inter-frequency RSRQ accuracy requirements for FR2	944

10.1A.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	944

10.1A.9.1.1	Absolute Accuracy of SS-RSRQ in FR2	944

10.1A.9.1.2	Relative Accuracy of SS-RSRQ in FR2	944

10.1A.10	 Intra-frequency SINR accuracy requirements for FR1	944

10.1A.10.1	Intra-frequency SS-SINR accuracy requirements in FR1	944

10.1A.10.1.1	Absolute SS-SINR Accuracy in FR1	944

10.1A.11	Intra-frequency SINR accuracy requirements for FR2	945

10.1A.11.1	Intra-frequency SS-SINR accuracy requirements in FR2	945

10.1A.11.1.1	Absolute SS-SINR Accuracy in FR2	945

10.1A.12 	Inter-frequency SINR accuracy requirements for FR1	945

10.1A.12.1	Inter-frequency SS-SINR accuracy requirements in FR1	945

10.1A.12.1.1	Aboslute Accuracy of SS-SINR in FR1	945

10.1A.12.1.2	Relative Accuracy of SS-SINR in FR1	946

10.1A.13	 Inter-frequency SINR accuracy requirements for FR2	947

10.1A.13.1	Inter-frequency SS-SINR accuracy requirements in FR2	947

10.1A.13.1.1	Aboslute Accuracy of SS-SINR in FR2	947

10.1A.13.1.2	Relative Accuracy of SS-SINR in FR2	947

10.1A.14	 L1-RSRP accuracy requirements for FR1	947

10.1A.14.1	SSB based L1-RSRP accuracy requirements	947

10.1A.14.1.1	Absolute Accuracy	947

10.1A.14.1.2	Relative Accuracy	948

10.1A.14.2	CSI-RS based L1-RSRP accuracy requirements	949

10.1A.14.2.1	Absolute Accuracy	949

10.1A.14.2.2	Relative Accuracy	950

10.1A.15	 L1-RSRP accuracy requirements for FR2	951

10.1A.15.1	SSB based L1-RSRP accuracy requirements	951

10.1A.15.1.1	Absolute Accuracy	951

10.1A.15.1.2	Relative Accuracy	951

10.1A.15.2	CSI-RS based L1-RSRP accuracy requirements	951

10.1A.15.2.1	Absolute Accuracy	951

10.1A.15.2.2	Relative Accuracy	951

10.2	E-UTRAN measurements	952

10.2.1	Introduction	952

10.2.2	E-UTRAN RSRP measurements	952

10.2.3	E-UTRAN RSRQ measurements	952

10.2.4	E-UTRAN RSTD measurements	952

10.2.5	E-UTRAN RS-SINR measurements	953

10.2.6	E-UTRAN RSRP measurements for CA/DC Idle Mode Measurements	953

10.2.7	E-UTRAN RSRQ measurements for CA/DC Idle Mode Measurements	953

10.3	UTRAN FDD Measurements	953

10.3.1	UTRAN FDD CPICH RSCP	954

10.3.2	UTRAN FDD CPICH Ec/No	954

10.4	V2X measurements	955

10.4.1	Introduction	955

10.4.2	Intra-frequency PSBCH-RSRP accuracy requirements for FR1	955

10.4.2.1	PSBCH-RSRP Absolute Accuracy	955

10.4.2.2	PSBCH-RSRP Relative Accuracy	955

10.4.3	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1	956

10.4.3.1	Absolute SL-RSSI Accuracy	956

10.4.4	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1	957

10.4.4.1	Absolute L1 SL-RSRP Accuracy	957

10.4.5	Intra-Frequency Discovery Signal Measurement Accuracy Requirements	957

10.4.5.1	Absolute Discovery Signal Measurement Accuracy	957

11	Void	958

12	V2X Requirements	958

12.1	Introduction	958

12.2	UE Transmit Timing	959

12.2.1	Introduction	959

12.2.2	GNSS as synchronization reference source	959

12.2.3	NR Cell as synchronization reference source	959

12.2.4	E-URTAN Cell as synchronization reference source	960

12.2.5	SyncRef UE as synchronization reference source	960

12.3	Initiation/Cease of SLSS Transmissions	960

12.3.1	Introduction	960

12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	960

12.3.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	961

12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	962

12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	962

12.4	Selection / Reselection of V2X Synchronization Reference Source	963

12.5	L1 SL-RSRP measurements	965

12.5.1	Introduction	965

12.5.2	SL-RSRP measurements	965

12.6	Congestion Control measurements	965

12.7	Interruption	966

12.7.1	Interruptions to WAN due to V2X Sidelink Communication	966

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change	966

12.7.3	Interruptions to WAN due to switching between E-UTRA V2X Sidelink and NR V2X Sidelink	968

12.7.4	Interruptions to WAN at transitions between active and non-active during SL-DRX	968

12.7.5	Interruptions to V2X sidelink at transitions between active and non-active during DRX	969

12.7.6	Interruptions to V2X sidelink due to Active BWP switching Requirement	969

12.7.7	Interruptions to WAN due to SyncRef UE detection and/or Sensing during SL DRX off duration	969

12.7.8	Interruptions at NR sidelink discovery configuration	969

12.8	Reliability of GNSS signal	970

12.9	Scheduling availability	970

12.9.1	Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink	970

12.9.2	Scheduling availability of UE switching between Uu uplink  and V2X sidelink	970

12.10	Selection / Reselection of relay UE	971

12.10.1	Introduction	971

12.10.2	Selection / Reselection of relay UE	971

13	Measurement Performance Requirements for NR gNB	972

13.1	UL-RTOA	972

13.1.1	Report mapping	972

13.2	gNB Rx-Tx time difference	974

13.2.1	Report mapping	974

13.2.2	Measurement Accuracy Requirements	976

13.2.2.1	Introduction	976

13.3	UL SRS RSRP measurement	977

13.3.1	Report mapping	977

13.3.2	Measurement accuracy requirements	978

13.3.2.1	Introduction	978

13.3.2.2	Requirements	978

13.4	AoA/ZoA	979

13.4.1	Report mapping	979

13.5	Timing advance (TADV)	980

13.5.1	Report mapping	980

13.6	UL SRS RSRPP measurement	981

13.6.1	Report mapping	981

13.7	gNB Rx-Tx time difference measurements for RTT-based PDC	983

13.7.1	Report mapping	983

13.7.2	Measurement Accuracy Requirements	983

13.7.2.1	Introduction	983

13.7.2.2	Requirements	983

Annex A (normative): Test Cases	967

A.1	Purpose of annex	967

A.2	Requirement classification for statistical testing	967

A.2.1	Types of requirements in TS 38.133	967

A.2.1.1	Time and delay requirements on UE higher layer actions	967

A.2.1.2	Measurements of power levels, relative powers and time	967

A.2.1.3	Implementation requirements	968

A.2.1.4	Physical layer timing requirements	968

A.2.1.5	Requirements under CCA	968

A.3	RRM test configurations	969

A.3.1	Reference measurement channels	969

A.3.1.1	PDSCH	969

A.3.1.1.1	FDD	969

A.3.1.1.2	TDD	970

A.3.1.2	CORESET for RMSI scheduling	973

A.3.1.2.1	FDD	973

A.3.1.2.2	TDD	974

A.3.1.3	CORESET for RMC scheduling	977

A.3.1.3.1	FDD	977

A.3.1.3.2	TDD	978

A.3.1.4	TDD UL/DL configuration	981

A.3.1A	Reference measurement channels under CCA	983

A.3.1A.1	PDSCH	983

A.3.1A.1.1	TDD	983

A.3.1A.2	CORESET for RMSI scheduling	984

A.3.1A.2.1	TDD	984

A.3.1A.3	CORESET for RMC scheduling	985

A.3.1A.3.1	TDD	985

A.3.1A.4	TDD UL/DL configuration	986

A.3.1A.5	RMC burst transmission model	986

A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)	986

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs	987

A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup	987

A.3.2.1.3	OCNG pattern 3: Generic OCNG pattern for unused REs in the same bandwidth as CORESET	988

A.3.2.1.4	OCNG pattern 4: Generic OCNG pattern for all unused REs outside SSB slot(s)	988

A.3.2.2	Void	989

A.3.3	Reference DRX configurations	989

A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms	989

A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms	990

A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity	990

A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms and TAT = Infinity	990

A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms and TAT = Infinity	991

A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = 500 ms	991

A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity	991

A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity	992

A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms	992

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms and TAT = 500 ms	992

A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity	993

A.3.3.12	DRX Configuration 12: DRX cycle = 640 ms and TAT = Infinity	993

A.3.3.13	DRX Configuration X1: DRX cycle = 80 ms and TAT = Infinity	993

A.3.4	Test Cases with Different Channel Bandwidths	994

A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths	994

A.3.4.1.1	Introduction	994

A.3.4.1.2	Principle of testing	994

A.3.5	Test Cases for Synchronous and Asynchronous DC Operations	994

A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	994

A.3.5.1.1	Introduction	994

A.3.5.1.2	Principle of Testing	994

A.3.6	Antenna configurations	994

A.3.6.1	Antenna configurations for FR1	994

A.3.6.1.1	Antenna connection for 4 Rx capable UEs	994

A.3.6.1.1.1	Introduction	994

A.3.6.1.1.2	Principle of testing	995

A.3.6.2	Antenna configurations for FR2	998

A.3.6A	Antenna configurations with unlicensed bands	998

A.3.6A.1	Antenna configurations for FR1	998

A.3.6A.1.1	Antenna connection for 4 Rx capable UEs	998

A.3.6A.1.1.1	Introduction	998

A.3.6A.1.1.2	Principle of testing	998

A.3.7	EN-DC test setup	1000

A.3.7.1	Introduction	1000

A.3.7.2	E-UTRAN Serving Cell Parameters	1000

A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	1000

A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	1001

A.3.7A	NR FR1-FR2 test setup	1002

A.3.7B	EN-DC test setup with unlicensed bands	1003

A.3.7B.1	Introduction	1003

A.3.7B.2	E-UTRAN Serving Cell Parameters	1003

A.3.7B.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	1003

A.3.7C	LTE-FR1/FR2 test setup	1004

A.3.7D	NE-DC test setup	1004

A.3.7D.1	Introduction	1004

A.3.7D.2	E-UTRAN Serving Cell Parameters	1004

A.3.7D.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	1004

A.3.7D.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	1004

A.3.8	PRACH configurations	1004

A.3.8.1	Introduction	1004

A.3.8.2	PRACH configurations in FR1	1005

A.3.8.2.1	FR1 PRACH configuration 1	1005

A.3.8.2.2	FR1 PRACH configuration 2	1005

A.3.8.2.3	FR1 PRACH configuration 3	1006

A.3.8.2.4	FR1 PRACH configuration 4	1007

A.3.8.3	PRACH configurations in FR2	1008

A.3.8.3.1	FR2 PRACH configuration 1	1008

A.3.8.3.2	FR2 PRACH configuration 2	1009

A.3.8.3.3	FR2 PRACH configuration 3	1010

A.3.8.3.4	FR2 PRACH configuration 4	1011

A.3.8A	PRACH configurations under CCA	1012

A.3.8A.1	Introduction	1012

A.3.8A.2	PRACH configurations in FR1	1012

A.3.8A.2.1	FR1 PRACH configuration 1 under CCA	1012

A.3.8A.2.2	FR1 PRACH configuration 2 under CCA	1013

A.3.9	BWP configurations	1014

A.3.9.1	Introduction	1014

A.3.9.2	Downlink BWP configurations	1015

A.3.9.2.1	Initial BWP	1015

A.3.9.2.2	Dedicated BWP	1015

A.3.9.3	Uplink BWP configurations	1016

A.3.9.3.1	Initial BWP	1016

A.3.9.3.2	Dedicated BWP	1016

A.3.9A	BWP configurations for RedCap	1016

A.3.9A.1	Introduction	1016

A.3.9A.2	Downlink BWP configurations	1017

A.3.9A.2.1	Dedicated BWP	1017

A.3.9A.3	Uplink BWP configurations	1017

A.3.9A.3.2	Dedicated BWP	1017

A.3.10	SSB Configurations	1018

A.3.10.1	SSB Configurations for FR1	1018

A.3.10.1.1	SSB pattern 1 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1018

A.3.10.1.5	SSB pattern 5 in FR1: SSB allocation for SSB SCS=15 kHz starting from odd SFN in 10 MHz	1020

A.3.10.1.6	SSB pattern 6 in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 40 MHz	1020

A.3.10.1.7	SSB pattern 7 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1021

A.3.10.1.8	SSB pattern 8 in FR1: SSB allocation for SSB SCS=30 kHz in 40 MHz	1021

A.3.10.2	SSB Configurations for FR2	1022

A.3.10.2.1	SSB pattern 1 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1022

A.3.10.2.2	SSB pattern 2 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1022

A.3.10.2.3	SSB pattern 3 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1023

A.3.10.2.4	SSB pattern 4 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1023

A.3.10.2.5	SSB pattern 5 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1024

A.3.10.2.6	SSB pattern 6 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1024

A.3.10.2.7	SSB pattern 7 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1025

A.3.10.2.8	SSB pattern 8 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1025

A.3.10.2.9	SSB pattern 9 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1026

A.3.10.2.10	SSB pattern 10 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1026

A.3.10A	SSB Configurations under CCA	1031

A.3.10A.1	SSB Configurations under CCA for FR1	1031

A.3.10A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz	1031

A.3.10A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz	1031

A.3.10A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1032

A.3.10A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1032

A.3.10B	SSB Configurations for RedCap	1033

A.3.10B.1	SSB Configurations for FR1	1033

A.3.10B.1.1	SSB pattern 1 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1033

A.3.10B.1.2	SSB pattern 2 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1033

A.3.10B.1.3	SSB pattern 3 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 20 MHz	1034

A.3.10B.1.4	SSB pattern 4 for RedCap in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1034

A.3.10B.1.5	SSB pattern 5 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1035

A.3.10B.1.6	SSB pattern 6 for RedCap in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1035

A.3.10B.1.7	SSB pattern 7 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1036

A.3.10B.2	SSB Configurations for FR2	1036

A.3.10B.2.1	SSB pattern 1 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1036

A.3.10B.2.2	SSB pattern 2 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1037

A.3.10B.2.3	SSB pattern 3 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1037

A.3.11	SMTC Configurations	1038

A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms	1038

A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms	1038

A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms	1038

A.3.11.4	SMTC pattern 4: SMTC period = 20 ms with SMTC duration = 1 ms	1038

A.3.11.5	SMTC pattern 5: SMTC period = 20 ms with SMTC duration = 5 ms	1039

A.3.11.6	SMTC pattern 6: SMTC period = 20 ms with SMTC duration = 5 ms	1039

A.3.11.7	SMTC pattern Y: SMTC period = 20 ms with SMTC duration = 5 ms	1039

A.3.11A	SMTC Configurations for RedCap	1039

A.3.11A.1	SMTC pattern 1 for RedCap: SMTC period = 40 ms with SMTC duration = 1 ms	1039

A.3.11A.2	SMTC pattern 2 for RedCap: SMTC period = 80 ms with SMTC duration = 1 ms	1039

A.3.11A.3	SMTC pattern 3 for RedCap: SMTC period = 40 ms with SMTC duration = 1 ms	1040

A.3.12	Test Cases with Different CC Configurations	1040

A.3.12.1 EN-DC Test Cases with Different EN-DC Configurations	1040

A.3.12.1.1	Introduction	1040

A.3.12.1.2	Principle of testing	1040

A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations	1040

A.3.12.2.1	Introduction	1040

A.3.12.2.2	Principle of testing	1040

A.3.13	Test Cases in SA and EN-DC Operations	1041

A.3.13.1	Introduction	1041

A.3.13.2	Principle of Testing	1041

A.3.13A	Test Cases involving E-UTRA/FR1 and FR2 carriers	1041

A.3.13A.1	Introduction	1041

A.3.13A.2	Principle of Testing in EN-DC	1041

A.3.13A.3	Principle of Testing in SA	1042

A.3.13A.4	Principle of Testing in E-UTRA	1042

A.3.13B	Test Cases for EN-DC and NE-DC Operations	1043

A.3.13B.1	Active BWP switch Test Cases for EN-DC and NE-DC Operations	1043

A.3.13B.1.1	Introduction	1043

A.3.13B.1.2	Principle of Testing	1043

A.3.13B.2	SFTD accuracy Test Cases for EN-DC and NE-DC Operations	1043

A.3.13B.2.1	Introduction	1043

A.3.13B.2.2	Principle of Testing	1043

A.3.14	CSI-RS configurations	1043

A.3.14.1	FDD	1043

A.3.14.2	TDD	1045

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases	1051

A.3.15.1	Setup 1: Single AoA in Rx beam peak direction	1051

A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction	1051

A.3.15.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	1051

A.3.15.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	1052

A.3.15.3	Setup 3: 2 AoAs	1052

A.3.15.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	1052

A.3.15.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	1052

A.3.15.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	1052

A.3.16	TCI State Configuration	1053

A.3.16.1	Introduction	1053

A.3.16.2	TCI states	1053

A.3.16A	Unified TCI State Configuration	1053

A.3.16A.1	Introduction	1053

A.3.16A.2	DLorJoint TCI states	1054

A.3.16A.3	UL TCI states	1054

A.3.17	Configurations of CSI-RS for tracking	1055

A.3.17.1	Configuration of CSI-RS for tracking for FR1	1055

A.3.17.1.1	FDD	1055

A.3.17.1.2	TDD	1057

A.3.17.2	Configuration of CSI-RS for tracking for FR2	1059

A.3.17.2.1	TDD	1059

A.3.18	Additional definitions related to OTA testing for FR2 RRM test cases	1060

A.3.18.1	Introduction	1060

A.3.18.2	PRACH Power Measurement	1060

A.3.19	Test applicability for DAPS handover	1060

A.3.19.1	Introduction	1060

A.3.19.2	Principle of testing	1060

A.3.20	MsgA configurations	1061

A.3.20.1	Introduction	1061

A.3.20.2	MsgA configurations in FR1	1061

A.3.20.2.1	FR1 MsgA configuration 1	1061

A.3.20.2.2	FR1 MsgA configuration 2	1062

A.3.20.3	MsgA configurations in FR2	1064

A.3.20.3.1	FR2 MsgA configuration 1	1064

A.3.20.3.2	FR2 MsgA configuration 2	1065

A.3.20A	MsgA configurations under CCA	1067

A.3.20A.1	Introduction	1067

A.3.20A.2	MsgA configurations in FR1	1067

A.3.20A.2.1	FR1 MsgA configuration 1 under CCA	1067

A.3.20A.2.2	FR1 MsgA configuration 2 under CCA	1068

A.3.21	V2X sidelink communication	1071

A.3.21.1	Introduction	1071

A.3.21.2	Reference resource pool configurations for V2X Sidelink Communication	1071

A.3.21.3	Reference measurement channels for V2X Sidelink Communication	1075

A.3.21.4	Reference SL-DRX configurations	1076

A.3.21.4.1	SL-DRX Configuration 1: SL-DRX cycle = 40 ms	1076

A.3.21.4.2	SL-DRX Configuration 2: SL-DRX cycle = 320 ms	1076

A.3.21.4.3	SL-DRX Configuration 3: SL-DRX cycle = 640 ms	1076

A.3.22	CSI-IM configurations	1076

A.3.22.1	FDD	1076

A.3.22.2	TDD	1077

A.3.23	Spatial Relation Configuration	1078

A.3.23.1	Introduction	1078

A.3.23.2	Spatial Relation	1079

A.3.24	SRS configuration	1080

A.3.25	Channel bandwidth (CBW) configurations	1082

A.3.25.1	DL UE specific CBW	1082

A.3.25.2	UL UE specific CBW	1083

A.3.26	CCA model	1083

A.3.26.1	Introduction	1083

A.3.26.2	CCA model for operation on a carrier frequency with CCA in FR1	1083

A.3.26.2.1	DL CCA model	1083

A.3.26.2.2	UL CCA model	1084

A.3.27	Test Cases with at Least One Cell on a Carrier Frequency with CCA	1085

A.3.27.1	Introduction	1085

A.3.27.2	 NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	1085

A.3.27.3	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	1085

A.3.27.4	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	1085

A.3.27.5	E-UTRA Standalone Tests with at Least One NR Cell under CCA	1085

A.3.28	Discovery Burst Transmission Window configuration under CCA	1086

A.3.28.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	1086

A.3.29	Testing principles for UE capable of only NR bands with shared spectrum access	1086

A.3.29.1	Introduction	1086

A.3.29.2	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	1086

A.3.29.3	Principle of testing for UE capable of SA operation with only NR bands with shared spectrum access	1086

A.3.30	CSI-RS configurations for RRM	1087

A.3.30.1	FDD	1087

A.3.30.2	TDD	1088

A.3.31	PRS Configurations	1090

A.3.31.1.	PRS Configurations for FR1	1090

A.3.31.1.1.	PRS pattern 1 in FR1: SCS=15 KHz	1090

A.3.31.1.2.	PRS pattern 2 in FR1: SCS=30 KHz	1091

A.3.31.2.	PRS Configurations for FR2	1091

A.3.31.2.1.	PRS pattern 1 in FR2: SCS=120 KHz	1091

A.3.32	NR sidelink discovery	1091

A.3.32.1	Introduction	1091

A.3.32.2	Reference resource pool configurations for NR Sidelink Discovery	1091

A.3.32.3	Principle of Testing	1092

A.3.33	PRS Processing Window (PPW) configurations	1092

A.3.34	Testing principles for test cases related to PRS measurements	1092

A.3.34.1	Introduction	1092

A.3.34.2	Test cases in RRC_INACTIVE state	1093

A.3.34.3	Test cases for PRS measurements with gaps in RRC_CONNECTED state	1093

A.3.34.4	Test cases for PRS measurements without gaps in RRC_CONNECTED state	1093

A.3.35	Testing principle for RedCap UE	1094

A.3.35.1	Introduction	1094

A.3.35.2	Principle of testing for FR1	1094

A.3.35.3	Principle of testing for FR2	1094

A.3.36	Testing principles for test cases related to Satellite access	1094

A.3.36.1	Introduction	1094

A.3.36.2	Principle of testing GSO and NGSO scenarios	1094

A.3.36.2	Principle of testing different RRM requirements	1095

A.4	EN-DC tests with all NR cells in FR1	1096

A.4.1	Void	1096

A.4.2	Void	1096

A.4.3	RRC_CONNECTED state mobility	1096

A.4.3.1	Void	1096

A.4.3.2	RRC Connection Mobility Control	1096

A.4.3.2.1	Void	1096

A.4.3.2.2	Random Access	1096

A.4.3.2.2.1	4-step RA type contention based random access test in FR1 for PSCell in EN-DC	1096

A.4.3.2.2.2	4-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	1100

A.4.3.2.2.3	2-step RA type contention based random access test in FR1 for PSCell in EN-DC	1104

A.4.3.2.2.4	2-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	1108

A.4.3.2.3	Void	1112

A.4.3.3	Handover with PSCell from EN-DC to EN-DC with known target PSCell in FR1	1112

A.4.3.3.1	Test Purpose and Environment	1112

A.4.3.3.2	Test Requirements	1118

A.4.4	Timing	1119

A.4.4.1	UE transmit timing	1119

A.4.4.1.1	NR UE Transmit Timing Test for FR1	1119

A.4.4.1.1.1	Test Purpose and environment	1119

A.4.4.1.1.2	Test requirements	1123

A.4.4.2	UE timer accuracy	1123

A.4.4.3	Timing advance	1123

A.4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	1123

A.4.4.3.1.1	Test Purpose and Environment	1123

A.4.4.3.1.2	Test Parameters	1123

A.4.4.3.1.3	Test Requirements	1127

A.4.5	Signaling characteristics	1127

A.4.5.1	Radio link Monitoring	1127

A.4.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	1128

A.4.5.1.1.1	Test Purpose and Environment	1128

A.4.5.1.1.2	Test Requirements	1132

A.4.5.1.2	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	1132

A.4.5.1.2.1	Test Purpose and Environment	1132

A.4.5.1.2.2	Test Requirements	1138

A.4.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	1138

A.4.5.1.3.1	Test Purpose and Environment	1138

A.4.5.1.3.2	Test Requirements	1143

A.4.5.1.4	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	1143

A.4.5.1.4.1	Test Purpose and Environment	1143

A.4.5.1.4.2	Test Requirements	1149

A.4.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	1149

A.4.5.1.5.1	Test Purpose and Environment	1149

A.4.5.1.5.2	Test Requirements	1154

A.4.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	1154

A.4.5.1.6.1	Test Purpose and Environment	1154

A.4.5.1.6.2	Test Requirements	1160

A.4.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	1160

A.4.5.1.7.1	Test Purpose and Environment	1160

A.4.5.1.7.2	Test Requirements	1165

A.4.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	1166

A.4.5.1.8.1	Test Purpose and Environment	1166

A.4.5.1.8.2	Test Requirements	1171

A.4.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	1171

A.4.5.1.9.1	Test Purpose and Environment	1171

A.4.5.2	Interruption	1177

A.4.5.2.1	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1177

A.4.5.2.1.1	Test Purpose and Environment	1177

A.4.5.2.1.2	Test Requirements	1181

A.4.5.2.2	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1181

A.4.5.2.2.1	Test Purpose and Environment	1181

A.4.5.2.2.2	Test Requirements	1185

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1185

A.4.5.2.3.1	Test Purpose and Environment	1185

A.4.5.2.3.2	Test Requirements	1190

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1191

A.4.5.2.4.1	Test Purpose and Environment	1191

A.4.5.2.4.2	Test Requirements	1195

A.4.5.2.5	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1196

A.4.5.2.5.1	Test Purpose and Environment	1196

A.4.5.2.5.2	Test Requirements	1200

A.4.5.2.6	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1200

A.4.5.2.6.1	Test Purpose and Environment	1200

A.4.5.2.6.2	Test Requirements	1204

A.4.5.2.7	Void	1204

A.4.5.2.8	E-UTRAN - NR FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	1204

A.4.5.2.8.1	Test Purpose and Environment	1204

A.4.5.2.8.2	Test Requirements	1207

A.4.5.2.9	E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching	1208

A.4.5.2.9.1	Test Purpose and Environment	1208

A.4.5.2.9.2	Test Requirements	1211

A.4.5.2.10	E-UTRAN – NR FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell	1212

A.4.5.2.10.1	Test Purpose and Environment	1212

A.4.5.2.10.2	Test Requirements	1215

A.4.5.2.11	E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching with 1 SRS symbol in a slot in synchronous EN-DC	1215

A.4.5.2.11.1	Test Purpose and Environment	1215

A.4.5.2.11.2	Test Requirements	1220

A.4.5.2.12	E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching in asynchronous EN-DC	1221

A.4.5.2.12.1	Test Purpose and Environment	1221

A.4.5.3	SCell Activation and Deactivation Delay	1229

A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1229

A.4.5.3.1.1	Test Purpose and Environment	1229

A.4.5.3.1.2	Test Requirements	1235

A.4.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 640ms SCell measurement cycle	1236

A.4.5.3.2.1	Test Purpose and Environment	1236

A.4.5.3.2.2	Test Requirements	1236

A.4.5.3.3	SCell Activation and deactivation of unknown SCell in FR1	1236

A.4.5.3.3.1	Test Purpose and Environment	1236

A.4.5.3.3.2	Test Requirements	1237

A.4.5.3.4	SCell Activation and deactivation of multiple unknown SCells in FR1 with single activation/deactivation command	1238

A.4.5.3.4.1	Test Purpose and Environment	1238

A.4.5.3.3.2	Test Requirements	1240

A.4.5.3.5	Direct SCell activation at SCell addition of known SCell in FR1	1240

A.4.5.3.5.1	Test Purpose and Environment	1240

A.4.5.3.5.2	Test Requirements	1248

A.4.5.3.3	Fast SCell Activation of known SCell in FR1 for 160ms SCell measurement cycle	1248

A.4.5.3.3.1	Test Purpose and Environment	1248

A.4.5.3.3.2	Test Requirements	1253

A.4.5.3.4	Fast SCell Activation of known SCell in FR1 for 640 ms SCell measurement cycle	1254

A.4.5.3.4.1	Test Purpose and Environment	1254

A.4.5.3.4.2	Test Requirements	1254

A.4.5.4	UE UL carrier RRC reconfiguration Delay	1255

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay	1255

A.4.5.4.1.1 Test Purpose and Environment	1255

A.4.5.4.1.2	Test Requirements	1265

A.4.5.5	Beam Failure Detection and Link recovery procedures	1265

A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	1265

A.4.5.5.1.1	Test Purpose and Environment	1265

A.4.5.5.1.2	Test Requirements	1271

A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	1271

A.4.5.5.2.1	Test Purpose and Environment	1271

A.4.5.5.2.2	Test Requirements	1277

A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1277

A.4.5.5.3.1	Test Purpose and Environment	1277

A.4.5.5.3.2	Test Requirements	1283

A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1283

A.4.5.5.4.1	Test Purpose and Environment	1283

A.4.5.5.4.2	Test Requirements	1289

A.4.5.5.5	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1289

A.4.5.5.5.1	Test Purpose and Environment	1289

A.4.5.5.5.2	Test Requirements	1295

A.4.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	1295

A.4.5.5.6.1	Test Purpose and Environment	1295

A.4.5.5.6.2	Test Requirements	1302

A.4.5.5.7	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	1302

A.4.5.5.7.1	Test Purpose and Environment	1302

A.4.5.5.7.2	Test Requirements	1308

A.4.5.5.8	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1308

A.4.5.5.8.1	Test Purpose and Environment	1308

A.4.5.5.8.2	Test Requirements	1315

A.4.5.6	Active BWP switch	1316

A.4.5.6.1	DCI-based and Timer-based Active BWP Switch	1316

A.4.5.6.1.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	1316

A.4.5.6.1.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	1321

A.4.5.6.2	RRC-based Active BWP Switch	1327

A.4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1331

A.4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	1331

A.4.5.6.3.1.1	Test Purpose and Environment	1331

A.4.5.6.3.1.2	Test Requirements	1337

A.4.5.6.4	Simultaneous RRC-based Active BWP Switch on multiple CCs	1337

A.4.5.6.4.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	1337

A.4.5.6.4.1.1	Test Purpose and Environment	1337

A.4.5.6.4.1.2	Test Requirements	1344

A.4.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	1344

A.4.5.6.4.2.1	Test Purpose and Environment	1344

A.4.5.6.4.2.2	Test Requirements	1354

A.4.5.6.5	SCell dormancy switch	1355

A.4.5.6.5.1	E-UTRAN – NR FR1 PSCell SCell dormancy switch of single FR1 SCell outside active time	1355

A.4.5.6.5.1.1	Test Purpose and Environment	1355

A.4.5.6.5.1.2	Test Requirements	1360

A.4.5.6.5.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	1360

A.4.5.6.5.2.1	Test Purpose and Environment	1360

A.4.5.6.5.2.2	Test Requirements	1366

A.4.5.7	PSCell addition and release delay	1366

A.4.5.7.1	Addition and Release Delay of known NR PSCell	1366

A.4.5.7.1.1	Test purpose and environment	1366

A.4.5.7.1.2	Test Requirements	1370

A.4.5.8	DL Interruptions at switching between two uplink carriers	1371

A.4.5.8.1	Test Purpose and Environment	1371

A.4.5.8.2	Test Requirements	1374

A.4.5.9	PSCell activation and deactivation delay	1375

A.4.5.9.1	PSCell activation and deactivation delay	1375

A.4.5.9.1.1	Test purpose and environment	1375

A.4.5.9.1.2	Test Requirements	1379

A.4.5.10	Conditional PSCell addition and release delay (FR1 EN-DC)	1380

A.4.5.10.1	Addition and Release Delay of PSCell	1380

A.4.5.10.1.1	Test purpose and environment	1380

A.4.5.10.1.2	Test Parameters	1380

A.4.5.10.1.3	Test Requirements	1384

A.4.6	Measurement procedure	1384

A.4.6.1	Intra-frequency Measurements	1384

A.4.6.1.1	EN-DC event triggered reporting tests without gap under non-DRX	1384

A.4.6.1.1.1	Test purpose and Environment	1384

A.4.6.1.1.2	Test parameters	1384

A.4.6.1.1.3	Test Requirements	1388

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX	1388

A.4.6.1.2.1	Test purpose and Environment	1388

A.4.6.1.2.2	Test parameters	1388

A.4.6.1.2.2	Test Requirements	1392

A.4.6.1.3	EN-DC event triggered reporting tests with per-UE gaps under non-DRX	1392

A.4.6.1.3.1	Test purpose and Environment	1392

A.4.6.1.3.2	Test parameters	1392

A.4.6.1.3.3	Test Requirements	1396

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX	1396

A.4.6.1.4.1	Test purpose and Environment	1396

A.4.6.1.4.2	Test parameters	1396

A.4.6.1.4.3	Test Requirements	1400

A.4.6.1.5	EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading	1400

A.4.6.1.5.1	Test purpose and Environment	1400

A.4.6.1.5.2	Test parameters	1400

A.4.6.1.5.3	Test Requirements	1402

A.4.6.1.6	EN-DC event triggered reporting tests with SSB index reading with per-UE gaps	1403

A.4.6.1.6.1	Test purpose and Environment	1403

A.4.6.1.6.2	Test parameters	1403

A.4.6.1.6.3	Test Requirements	1405

A.4.6.1.7	EN-DC event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	1406

A.4.6.1.7.1	Test purpose and Environment	1406

A.4.6.1.7.2	Test parameters	1406

A.4.6.1.7.3	Test Requirements	1410

A.4.6.1.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used for UE configured with highSpeedMeasCA-Scell-r17	1410

A.4.6.1.8.1	Test Purpose and Environment	1410

A.4.6.1.8.2	Test Requirements	1415

A.4.6.2	Inter-frequency Measurements	1416

A.4.6.2.1	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	1416

A.4.6.2.1.1	Test Purpose and Environment	1416

A.4.6.2.1.2	Test Requirements	1420

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	1420

A.4.6.2.2.1	Test Purpose and Environment	1420

A.4.6.2.2.2	Test Requirements	1424

A.4.6.2.3	Void	1425

A.4.6.2.4	Void	1425

A.4.6.2.5	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	1425

A.4.6.2.5.1	Test Purpose and Environment	1425

A.4.6.2.5.2	Test Requirements	1429

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	1429

A.4.6.2.6.1	Test Purpose and Environment	1429

A.4.6.2.6.2	Test Requirements	1434

A.4.6.2.7	Void	1435

A.4.6.2.8	Void	1435

A.4.6.2.9	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used for UE configured with highSpeedMeasInterFreq-r17	1435

A.4.6.2.9.1	Test Purpose and Environment	1435

A.4.6.2.9.2	Test Requirements	1438

A.4.6.3	Void	1439

A.4.6.4	L1-RSRP measurement for beam reporting	1439

A.4.6.4.1	SSB based L1-RSRP measurement when DRX is not used	1439

A.4.6.4.1.1	Test Purpose and Environment	1439

A.4.6.4.1.2	Test parameters	1439

A.4.6.4.1.3	Test Requirements	1442

A.4.6.4.2	SSB based L1-RSRP measurement when DRX is used	1442

A.4.6.4.2.1	Test Purpose and Environment	1442

A.4.6.4.2.2	Test parameters	1443

A.4.6.4.2.3	Test Requirements	1446

A.4.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	1446

A.4.6.4.3.1	Test Purpose and Environment	1446

A.4.6.4.3.2	Test parameters	1447

A.4.6.4.3.3	Test Requirements	1450

A.4.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	1450

A.4.6.4.4.1	Test Purpose and Environment	1450

A.4.6.4.4.2	Test parameters	1451

A.4.6.4.4.3	Test Requirements	1453

A.4.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	1453

A.4.6.4.5.1	Test Purpose and Environment	1453

A.4.6.4.5.2	Test parameters	1454

A.4.6.4.5.3	Test Requirements	1457

A.4.6.5	CLI measurements	1457

A.4.6.5.1	SRS-RSRP measurement with non-DRX	1457

A.4.6.5.1.1	Test Purpose and Environment	1457

A.4.6.5.1.2	Test Parameters	1458

A.4.6.5.1.3	Test Requirements	1461

A.4.6.5.2	CLI-RSSI measurement with non-DRX	1461

A.4.6.5.2.1	Test Purpose and Environment	1461

A.4.6.5.2.2	Test Parameters	1462

A.4.6.5.2.3	Test Requirements	1463

A.4.6.6.1.2	Test Requirements	1468

A.4.6.7	L1-SINR measurement for beam reporting	1468

A.4.6.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	1471

A.4.6.7.2.1	Test Purpose and Environment	1471

A.4.6.7.2.2	Test parameters	1471

A.4.6.7.2.3	Test Requirements	1474

A.4.6.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	1474

A.4.6.7.3.1	Test Purpose and Environment	1474

A.4.6.7.3.2	Test parameters	1474

A.4.6.7.3.3	Test Requirements	1476

A.4.6.8	CSI-RS based intra-frequency Measurement	1477

A.4.6.8.1	EN-DC event triggered reporting tests without gap under DRX	1477

A.4.6.8.1.1	Test purpose and Environment	1477

A.4.6.8.1.2	Test Requirements	1481

A.4.6.9	CSI-RS based inter-frequency Measurement	1481

A.4.6.9.1	EN-DC event triggered reporting tests for FR1 cell when non-DRX is used	1481

A.4.6.9.1.1	Test Purpose and Environment	1481

A.4.6.9.1.2	Test Requirements	1485

A.4.7	Measurement Performance requirements	1486

A.4.7.1	SS-RSRP	1486

A.4.7.1.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1486

A.4.7.1.1.1	Test Purpose and Environment	1486

A.4.7.1.1.2	Test parameters	1486

A.4.7.1.1.3	Test Requirements	1490

A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1490

A.4.7.1.2.1	Test Purpose and Environment	1490

A.4.7.1.2.2	Test parameters	1491

A.4.7.1.2.3	Test Requirements	1495

A.4.7.1.3	Void	1495

A.4.7.2	SS-RSRQ	1495

A.4.7.2.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1495

A.4.7.2.1.1	Test Purpose and Environment	1495

A.4.7.2.1.2	Test Parameters	1496

A.4.7.2.1.3	Test Requirements	1501

A.4.7.2.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1501

A.4.7.2.2.1	Test Purpose and Environment	1501

A.4.7.2.2.2	Test Parameters	1501

A.4.7.2.2.3	Test Requirements	1506

A.4.7.3	SS-SINR	1506

A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1506

A.4.7.3.1.1	Test Purpose and Environment	1506

A.4.7.3.1.2	Test Parameters	1506

A.4.7.3.1.3	Test Requirements	1510

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1510

A.4.7.3.2.1	Test Purpose and Environment	1510

A.4.7.3.2.2	Test Parameters	1510

A.4.7.3.2.3	Test Requirements	1515

A.4.7.4	L1-RSRP measurement for beam reporting	1515

A.4.7.4.1	SSB based L1-RSRP measurement	1515

A.4.7.4.1.1	Test Purpose and Environment	1515

A.4.7.4.1.2	Test parameters	1516

A.4.7.4.1.3	Test Requirements	1519

A.4.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1519

A.4.7.4.2.1	Test Purpose and Environment	1519

A.4.7.4.2.2	Test parameters	1520

A.4.7.4.2.3	Test Requirements	1523

A.4.7.5	SFTD accuracy	1523

A.4.7.5.1	SFTD accuracy	1523

A.4.7.5.1.1	Test Purpose and Environment	1523

A.4.7.5.1.2	Test Parameters	1523

A.4.7.5.1.3	Test Requirements	1528

A.4.7.5.2	Void	1528

A.4.7.5.3	Void	1528

A.4.7.6	CLI measurements	1528

A.4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	1528

A.4.7.6.1.1	Test Purpose and Environment	1528

A.4.7.6.1.2	Test parameters	1529

A.4.7.6.1.3	Test Requirements	1534

A.4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	1534

A.4.7.6.2.1	Test Purpose and Environment	1534

A.4.7.6.2.2	Test parameters	1535

A.4.7.6.2.3	Test Requirements	1538

A.4.7.7	L1-SINR measurement for beam reporting	1538

A.4.7.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1542

A.4.7.7.2.1	Test Purpose and Environment	1542

A.4.7.7.2.2	Test parameters	1542

A.4.7.7.2.3	Test Requirements	1546

A.4.7.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1546

A.4.7.7.3.1	Test Purpose and Environment	1546

A.4.7.7.3.2	Test parameters	1546

A.4.7.7.3.3	Test Requirements	1549

A.4.7.8	CSI-RSRP	1549

A.4.7.8.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1549

A.4.7.8.1.1	Test Purpose and Environment	1549

A.4.7.8.1.2	Test parameters	1550

A.4.7.8.1.3	Test Requirements	1555

A.4.7.8.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1555

A.4.7.8.2.1	Test Purpose and Environment	1555

A.4.7.8.2.2	Test parameters	1555

A.4.7.8.2.3	Test Requirements	1560

A.4.7.9	CSI-RSRQ	1560

A.4.7.9.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1560

A.4.7.9.1.1	Test Purpose and Environment	1560

A.4.7.9.1.2	Test Parameters	1560

A.4.7.9.1.3	Test Requirements	1565

A.4.7.9.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1565

A.4.7.9.2.1	Test Purpose and Environment	1565

A.4.7.9.2.2	Test Parameters	1565

A.4.7.9.2.3	Test Requirements	1570

A.4.7.10	CSI-SINR	1570

A.4.7.10.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1570

A.4.7.10.1.1	Test Purpose and Environment	1570

A.4.7.10.1.2	Test Parameters	1571

A.4.7.10.1.3	Test Requirements	1575

A.4.7.10.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1575

A.4.7.10.2.1	Test Purpose and Environment	1575

A.4.7.10.2.2	Test Parameters	1576

A.4.7.10.2.3	Test Requirements	1580

A.4.8	Void	1580

A.4A	NE-DC test with all NR cells in FR1	1581

A.4A.1	Signaling characteristics	1581

A.4A.1.1	E-UTRAN PSCell addition	1581

A.4A.1.1.1	Test purpose and environment	1581

A.4A.1.1.2	Test Requirements	1585

A.4A.1.2	Active BWP switch	1586

A.4A.1.2.1	E-UTRAN PSCell – NR PCell FR1 DCI-based and Timer-based DL active BWP switch in non-DRX in synchronous NE-DC	1586

A.4A.1.2.1.1	Test Purpose and Environment	1586

A.4A.1.2.1.2	Test Requirements	1590

A.4A.1.3	Intra-frequency handover with E-UTRAN PSCell	1591

A.4A.1.3.1	Test purpose and environment	1591

A.4A.1.3.2	Test Requirements	1595

A.4A.1.4	Handover with PSCell from NE-DC to NE-DC with unknown target PSCell	1596

A.4A.1.4.1	Test Purpose and Environment	1596

A.4A.1.4.2	Test Parameters	1596

A.4A.1.4.3	Test Requirements	1601

A.4A.1.4.3.1	Test Requirements for NR HO	1601

A.4A.1.4.3.2	Test Requirements for LTE PSCell Change	1601

A.4A.2	Measurement performance	1602

A.4A.2.1	SFTD accuracy	1602

A.4A.2.1.1	SFTD accuracy	1602

A.4A.2.1.1.1	Test Purpose	1602

A.4A.2.1.1.2	Test Environment	1602

A.4A.2.1.1.3	Test Requirements	1605

A.5	EN-DC tests with one or more NR cells in FR2	1606

A.5.1	Void	1606

A.5.2	Void	1606

A.5.3	RRC_CONNECTED state mobility	1606

A.5.3.1	Void	1606

A.5.3.2	RRC Connection Mobility Control	1606

A.5.3.2.1	Void	1606

A.5.3.2.2	Random Access	1606

A.5.3.2.2.1	4-step RA type c ontention based random access test in FR2 for PSCell/SCell in EN-DC	1606

A.5.3.2.2.2	4-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	1609

A.5.3.2.2.3	2-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	1615

A.5.3.2.2.4	2-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	1618

A.5.3.2.3	Void	1622

A.5.3.3	Handover with PSCell with known FR2 target PSCell	1622

A.5.3.3.1	Test purpose and environment	1622

A.5.3.3.2	Test Requirements	1626

A.5.5.3.3	PUCCH SCell Activation and deactivation of known SCell in FR2	1627

A.5.5.3.3.1	Test Purpose and Environment	1627

A.5.5.3.3.2	Test Requirements	1630

A.5.5.3.4	PUCCH SCell Activation and deactivation of unknown SCell in FR2	1631

A.5.5.3.4.1	Test Purpose and Environment	1631

A.5.5.3.4.2	Test Requirements	1634

A.5.5.3.5	Multiple SCell activation and deactivation of one known PUCCH SCell and one unknown SCell in FR2	1635

A.5.5.3.5.1	Test Purpose and Environment	1635

A.5.5.3.5.2	Test Requirements	1638

A.5.5.3.6	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR2 in non-DRX	1639

A.5.5.3.6.1	Test Purpose and Environment	1639

A.5.5.3.6.2	Test Requirements	1642

A.5.4	Timing	1643

A.5.4.1	UE transmit timing	1643

A.5.4.1.1	NR UE Transmit Timing Test for FR2	1643

A.5.4.1.1.1	Test Purpose and environment	1643

A.5.4.1.1.2	Test requirements	1646

A.5.4.2	UE timer accuracy	1647

A.5.4.3	Timing advance	1647

A.5.4.3.1 EN-DC FR2 timing advance adjustment accuracy	1647

A.5.4.3.1.1 Test Purpose and Environment	1647

A.5.4.3.1.2 Test Parameters	1647

A.5.4.3.1.3	Test Requirements	1651

A.5.5	Signaling characteristics	1651

A.5.5.1	Radio link Monitoring	1651

A.5.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	1651

A.5.5.1.1.1	Test Purpose and Environment	1651

A.5.5.1.1.2	Test Requirements	1655

A.5.5.1.2	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	1656

A.5.5.1.2.1	Test Purpose and Environment	1656

A.5.5.1.2.2	Test Requirements	1659

A.5.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	1660

A.5.5.1.3.1	Test Purpose and Environment	1660

A.5.5.1.3.2	Test Requirements	1664

A.5.5.1.4	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	1664

A.5.5.1.4.1	Test Purpose and Environment	1664

A.5.5.1.4.2	Test Requirements	1668

A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	1668

A.5.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	1672

A.5.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1676

A.5.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1681

A.5.5.1.8.2	Test Requirements	1685

A.5.5.1.9	EN-DC Radio Link Monitoring UE Scheduling Restrictions on FR2	1686

A.5.5.1.9.1	Test Purpose and Environment	1686

A.5.5.1.9.2	Test Requirements	1688

A.5.5.1.10	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	1688

A.5.5.1.10.1	Test Purpose and Environment	1688

A.5.5.1.10.2	Test Requirements	1691

A.5.5.2	Interruption	1691

A.5.5.2.1	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1691

A.5.5.2.1.1	Test Purpose and Environment	1691

A.5.5.2.1.2	Test Requirements	1694

A.5.5.2.2	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1694

A.5.5.2.2.1	Test Purpose and Environment	1694

A.5.5.2.2.2	Test Requirements	1697

A.5.5.2.3	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1697

A.5.5.2.3.1	Test Purpose and Environment	1697

A.5.5.2.3.2	Test Requirements	1701

A.5.5.2.4	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1701

A.5.5.2.4.1	Test Purpose and Environment	1701

A.5.5.2.4.2	Test Requirements	1704

A.5.5.2.5	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1705

A.5.5.2.5.1	Test Purpose and Environment	1705

A.5.5.2.5.2	Test Requirements	1708

A.5.5.2.6	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1708

A.5.5.2.6.1	Test Purpose and Environment	1708

A.5.5.2.6.2	Test Requirements	1711

A.5.5.2.7	E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching	1711

A.5.5.2.7.1	Test Purpose and Environment	1711

A.5.5.2.7.2	Test Requirements	1715

A.5.5.2.8 E-UTRAN – NR FR2 interruptions at NR SRS carrier based switching	1715

A.5.5.2.8.1 Test Purpose and Environment	1715

A.5.5.2.8.3	Test Requirements	1717

A.5.5.3	SCell Activation and Deactivation Delay	1717

A.5.5.3.1	SCell Activation and deactivation of SCell in FR2 intra-band	1717

A.5.5.3.1.1	Test Purpose and Environment	1717

A.5.5.3.1.2	Test Requirements	1719

A.5.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1719

A.5.5.3.2.1	Test Purpose and Environment	1719

A.5.5.3.2.2	Test Requirements	1723

A.5.5.3.3	Void	1723

A.5.5.3.4	Void	1723

A.5.5.3.5	SCell Activation and deactivation of SCell in FR2	1723

A.5.5.3.5.1	Test Purpose and Environment	1723

A.5.5.3.5.2	Test Requirements	1726

A.5.5.3.6	Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2	1727

A.5.5.3.6.1	Test Purpose and Environment	1727

A.5.5.3.6.2	Test Requirements	1731

A.5.5.3.7	Direct SCell activation at SCell addition of known SCell in FR2	1732

A.5.5.3.7.1	Test Purpose and Environment	1732

A.5.5.3.7.2	Test Requirements	1735

A.5.5.3.8	Fast SCell Activation of SCell in FR2 intra-band	1735

A.5.5.3.8.1	Test Purpose and Environment	1735

A.5.5.3.8.2	Test Requirements	1739

A.5.5.4	Void	1740

A.5.5.5	Beam Failure Detection and Link recovery procedures	1740

A.5.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1740

A.5.5.5.1.1	Test Purpose and Environment	1740

A.5.5.5.1.2	Test Requirements	1744

A.5.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode	1744

A.5.5.5.2.1	Test Purpose and Environment	1744

A.5.5.5.2.2	Test Requirements	1748

A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1748

A.5.5.5.3.1	Test Purpose and Environment	1748

A.5.5.5.3.2	Test Requirements	1752

A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1752

A.5.5.5.4.1	Test Purpose and Environment	1752

A.5.5.5.4.2	Test Requirements	1756

A.5.5.5.5	EN-DC scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1756

A.5.5.5.5.1	Test Purpose and Environment	1756

A.5.5.5.5.2	Test Requirements	1760

A.5.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1760

A.5.5.5.6.1	Test Purpose and Environment	1760

A.5.5.5.6.2	Test Requirements	1764

A.5.5.5.7	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	1764

A.5.5.5.7.1	Test Purpose and Environment	1764

A.5.5.5.7.2	Test Requirements	1768

A.5.5.5.8	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1768

A.5.5.5.8.1	Test Purpose and Environment	1768

A.5.5.5.8.2	Test Requirements	1774

A.5.5.5.9	Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode for UE fulfilling relaxed measurement criterion	1774

A.5.5.5.9.1	Test Purpose and Environment	1774

A.5.5.5.9.2	Test Requirements	1778

A.5.5.6	Active BWP switch	1778

A.5.5.6.1	DCI-based and Timer-based Active BWP Switch	1778

A.5.5.6.1.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1778

A.5.5.6.1.1.1	Test Purpose and Environment	1778

A.5.5.6.1.1.2	Test Requirements	1782

A.5.5.6.1.2	E-UTRAN – NR PSCell FR2 with FR2 SCell DL active BWP switch in non-DRX in synchronous EN-DC	1782

A.5.5.6.2	RRC-based Active BWP Switch	1787

A.5.5.6.2.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1787

A.5.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1791

A.5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	1791

A.5.5.6.3.1.1	Test Purpose and Environment	1791

A.5.5.6.3.1.2	Test Requirements	1795

A.5.5.6.4	SCell dormancy switch	1795

A.5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time	1795

A.5.5.6.4.1.1	Test Purpose and Environment	1795

A.5.5.6.4.1.2	Test Requirements	1799

A.5.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR2 SCells outside active time	1799

A.5.5.6.4.2.1	Test Purpose and Environment	1799

A.5.5.6.4.2.2	Test Requirements	1806

A.5.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1806

A.5.5.6.5.1	E-UTRAN – NR PSCell FR2  and NR SCell FR2 DL active BWP switch on multiple CCs with non-DRX in synchronous EN-DC	1806

A.5.5.7	PSCell addition and release delay	1809

A.5.5.7.1	Addition and Release Delay of NR PSCell	1809

A.5.5.7.1.1	Test purpose and environment	1809

A.5.5.7.1.2	Test Requirements	1812

A.5.5.8	Active TCI state switch delay	1813

A.5.5.8.1	MAC-CE based active TCI state switch	1813

A.5.5.8.1.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1813

A.5.5.8.1.1.1	Test Purpose and Environment	1813

A.5.5.8.1.1.2	Test Requirements	1816

A.5.5.8.2	RRC based active TCI state switch	1816

A.5.5.8.2.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1816

A.5.5.8.2.1.1	Test Purpose and Environment	1816

A.5.5.8.2.1.2	Test Requirements	1820

A.5.5.9	Uplink spatial relation switch delay	1820

A.5.5.9.1	MAC-CE based uplink spatial relation switch	1820

A.5.5.9.1.1	E-UTRAN – NR PSCell FR2 uplink spatial relation switch for a known spatial relation	1820

A.5.5.9.1.1.1	Test Purpose and Environment	1820

A.5.5.9.1.1.2	Test Requirements	1823

A.5.5.9.2	RRC based spatial relation switch	1823

A.5.5.9.2.1	E-UTRAN – NR PSCell FR2 spatial relation switch associated with a known DL-RS	1823

A.5.5.9.2.1.1	Test Purpose and Environment	1823

A.5.5.9.2.1.2	Test Requirements	1826

A.5.5.10	UE specific CBW change	1826

A.5.5.10.1	UE specific CBW change on FR2 NR PSCell	1826

A.5.5.10.1.1	Test Purpose and Environment	1826

A.5.5.10.1.2	Test Requirements	1829

A.5.5.11	Unified TCI state switch delay	1830

A.5.5.11.1	MAC-CE based active joint TCI state switch	1830

A.5.5.11.1.1	E-UTRAN – NR PSCell FR2 active joint TCI state switch for a known TCI state	1830

A.5.5.11.1.1.1	Test Purpose and Environment	1830

A.5.5.11.1.1.2	Test parameters	1830

A.5.5.11.1.1.3	Test Requirements	1833

A.5.5.11.2	MAC-CE based active uplink TCI state switch	1833

A.5.5.11.2.1	E-UTRAN – NR PSCell FR2 active uplink TCI state switch for a known TCI state	1833

A.5.5.11.2.1.1	Test Purpose and Environment	1833

A.5.5.11.2.1.2	Test parameters	1834

A.5.5.11.2.1.3	Test Requirements	1836

A.5.5.11.3	MAC-CE based active downlink TCI state switch	1836

A.5.5.11.3.1	E-UTRAN – NR PSCell FR2 downlink TCI state switch to cell with additional PCI for a known TCI state	1836

A.5.5.11.3.1.1	Test Purpose and Environment	1836

A.5.5.11.3.1.2	Test Parameters	1837

A.5.5.11.3.1.3	Test Requirements	1841

A.5.5.12	PSCell activation and deactivation delay	1841

A.5.5.12.1	PSCell activation and deactivation delay	1841

A.5.5.12.1.1	Test purpose and environment	1841

A.5.5.12.1.2	Test Requirements	1844

A.5.5.13	Conditional PSCell addition and release delay	1845

A.5.5.13.1	Addition and Release Delay of NR PSCell	1845

A.5.5.13.1.1	Test purpose and environment	1845

A.5.5.13.1.2	Test Requirements	1848

A.5.6	Measurement procedure	1849

A.5.6.1	Intra-frequency Measurements	1849

A.5.6.1.1	EN-DC event triggered reporting test without gap under non-DRX	1849

A.5.6.1.1.1	Test purpose and Environment	1849

A.5.6.1.1.2	Test Requirements	1852

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX	1852

A.5.6.1.2.1	Test purpose and Environment	1852

A.5.6.1.2.2	Test Requirements	1854

A.5.6.1.3	EN-DC event triggered reporting test with per-UE gaps under non-DRX	1855

A.5.6.1.3.1	Test purpose and Environment	1855

A.5.6.1.3.2	Test Requirements	1859

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX	1859

A.5.6.1.4.1	Test purpose and Environment	1859

A.5.6.1.4.2	Test Requirements	1862

A.5.6.2	Inter-frequency Measurements	1863

A.5.6.2.1 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1863

A.5.6.2.1.1	Test Purpose and Environment	1863

A.5.6.2.1.2	Test Requirements	1866

A.5.6.2.2 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1866

A.5.6.2.2.1	Test Purpose and Environment	1866

A.5.6.2.2.2	Test Requirements	1870

A.5.6.2.3 	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1870

A.5.6.2.3.1	Test Purpose and Environment	1870

A.5.6.2.3.2	Test Requirements	1874

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1874

A.5.6.2.4.1	Test Purpose and Environment	1874

A.5.6.2.4.2	Test Requirements	1878

A.5.6.2.5	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1878

A.5.6.2.5.1	Test Purpose and Environment	1878

A.5.6.2.5.2	Test Requirements	1883

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1883

A.5.6.2.6.1	Test Purpose and Environment	1883

A.5.6.2.6.2	Test Requirements	1887

A.5.6.2.7	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1887

A.5.6.2.7.1	Test Purpose and Environment	1887

A.5.6.2.7.2	Test Requirements	1892

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1892

A.5.6.2.8.1	Test Purpose and Environment	1892

A.5.6.2.8.2	Test Requirements	1897

A.5.6.3	L1-RSRP measurement for beam reporting	1898

A.5.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1898

A.5.6.3.1.1	Test Purpose and Environment	1898

A.5.6.3.1.2	Test parameters	1898

A.5.6.3.1.3	Test Requirements	1900

A.5.6.3.2	SSB based L1-RSRP measurement when DRX is used	1900

A.5.6.3.2.1	Test Purpose and Environment	1900

A.5.6.3.2.2	Test parameters	1901

A.5.6.3.2.3	Test Requirements	1903

A.5.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	1903

A.5.6.3.3.1	Test Purpose and Environment	1903

A.5.6.3.3.2	Test parameters	1904

A.5.6.3.3.3	Test Requirements	1906

A.5.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	1906

A.5.6.3.4.1	Test Purpose and Environment	1906

A.5.6.3.4.2	Test parameters	1907

A.5.6.3.4.3	Test Requirements	1909

A.5.6.4	CLI measurements	1910

A.5.6.4.1	SRS-RSRP measurement with DRX	1910

A.5.6.4.1.1	Test Purpose and Environment	1910

A.5.6.4.1.2	Test Parameters	1910

A.5.6.4.1.3	Test Requirements	1912

A.5.6.4.2	CLI-RSSI measurement with DRX	1912

A.5.6.4.2.1	Test Purpose and Environment	1912

A.5.6.4.2.2	Test Parameters	1913

A.5.6.4.2.3	Test Requirements	1915

A.5.6.5	Measurements with autonomous gaps	1915

A.5.6.5.1 	EN-DC inter-frequency CGI identification of NR neighbor cell in FR2	1915

A.5.6.5.1.1	Test Purpose and Environment	1915

A.5.6.5.1.2	Test Requirements	1918

A.5.6.6	L1-SINR measurement for beam reporting	1919

A.5.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	1921

A.5.6.6.2.1	Test Purpose and Environment	1921

A.5.6.6.2.2	Test parameters	1922

A.5.6.6.2.3	Test Requirements	1925

A.5.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	1925

A.5.6.6.3.1	Test Purpose and Environment	1925

A.5.6.6.3.2	Test parameters	1926

A.5.6.6.3.3	Test Requirements	1928

A.5.6.7	CSI-RS based Intra-frequency Measurements	1928

A.5.6.7.1	EN-DC event triggered reporting test without gap under non-DRX	1928

A.5.6.7.1.1	Test purpose and Environment	1928

A.5.6.7.1.2	Test Requirements	1931

A.5.6.8	CSI-RS based Inter-frequency Measurements	1931

A.5.6.8.1 	EN-DC event triggered reporting tests for NR FR2 cell when DRX is used	1931

A.5.6.8.1.1	Test Purpose and Environment	1931

A.5.6.8.1.2	Test Requirements	1936

A.5.7	Measurement Performance requirements	1936

A.5.7.1	SS-RSRP	1936

A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1936

A.5.7.1.1.1	Test Purpose and Environment	1936

A.5.7.1.1.2	Test parameters	1937

A.5.7.1.1.3	Test Requirements	1939

A.5.7.1.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1940

A.5.7.1.2.1	Test Purpose and Environment	1940

A.5.7.1.2.2	Test parameters	1940

A.5.7.1.2.3	Test Requirements	1944

A.5.7.1.3	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	1945

A.5.7.1.3.1	Test Purpose and Environment	1945

A.5.7.1.3.2	Test parameters	1945

A.5.7.1.3.3	Test Requirements	1948

A.5.7.2	SS-RSRQ	1949

A.5.7.2.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1949

A.5.7.2.1.1	Test Purpose and Environment	1949

A.5.7.2.1.2	Test Parameters	1949

A.5.7.2.1.3	Test Requirements	1952

A.5.7.2.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1952

A.5.7.2.2.1	Test Purpose and Environment	1952

A.5.7.2.2.2	Test Parameters	1952

A.5.7.2.2.3	Test Requirements	1954

A.5.7.3	SS-SINR	1955

A.5.7.3.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1955

A.5.7.3.1.1	Test Purpose and Environment	1955

A.5.7.3.1.2	Test Parameters	1955

A.5.7.3.1.3	Test Requirements	1958

A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1958

A.5.7.3.2.1	Test Purpose and Environment	1958

A.5.7.3.2.2	Test Parameters	1958

A.5.7.3.2.3	Test Requirements	1960

A.5.7.4	L1-RSRP measurement for beam reporting	1960

A.5.7.4.1	SSB based L1-RSRP measurement	1960

A.5.7.4.1.1	Test Purpose and Environment	1960

A.5.7.4.1.2	Test parameters	1961

A.5.7.4.1.3	Test Requirements	1963

A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1964

A.5.7.4.2.1	Test Purpose and Environment	1964

A.5.7.4.2.2	Test parameters	1964

A.5.7.4.2.3	Test Requirements	1966

A.5.7.5	CLI measurements	1967

A.5.7.5.1	EN-DC SRS-RSRP measurement accuracy with FR2 serving cell	1967

A.5.7.5.1.1	Test Purpose and Environment	1967

A.5.7.5.1.2	Test parameters	1967

A.5.7.5.1.3	Test Requirements	1970

A.5.7.5.2	EN-DC CLI-RSSI measurement accuracy with FR2 serving cell	1971

A.5.7.5.2.1	Test Purpose and Environment	1971

A.5.7.5.2.2	Test parameters	1971

A.5.7.5.2.3	Test Requirements	1973

A.5.7.6	L1-SINR measurement for beam reporting	1974

A.5.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1976

A.5.7.6.2.1	Test Purpose and Environment	1976

A.5.7.6.2.2	Test parameters	1977

A.5.7.6.2.3	Test Requirements	1979

A.5.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1980

A.5.7.6.3.1	Test Purpose and Environment	1980

A.5.7.6.3.2	Test parameters	1980

A.5.7.6.3.3	Test Requirements	1982

A.5.7.7	CSI-RSRP	1982

A.5.7.7.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1982

A.5.7.7.1.1	Test Purpose and Environment	1982

A.5.7.7.1.2	Test parameters	1983

A.5.7.7.1.3	Test Requirements	1985

A.5.7.7.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1986

A.5.7.7.2.1	Test Purpose and Environment	1986

A.5.7.7.2.2	Test parameters	1986

A.5.7.7.2.3	Test Requirements	1990

A.5.7.8	CSI-RSRQ	1991

A.5.7.8.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	1991

A.5.7.8.1.1	Test Purpose and Environment	1991

A.5.7.8.1.2	Test Parameters	1991

A.5.7.8.1.3	Test Requirements	1993

A.5.7.8.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1993

A.5.7.8.2.1	Test Purpose and Environment	1993

A.5.7.8.2.2	Test Parameters	1993

A.5.7.8.2.3	Test Requirements	1995

A.5.7.9	CSI-SINR	1995

A.5.7.9.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1995

A.5.7.9.1.1	Test Purpose and Environment	1995

A.5.7.9.1.2	Test Parameters	1995

A.5.7.9.1.3	Test Requirements	1998

A.5.7.9.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1998

A.5.7.9.2.1	Test Purpose and Environment	1998

A.5.7.9.2.2	Test Parameters	1998

A.5.7.9.2.3	Test Requirements	2001

A.5.8	Void	2001

A.6	NR standalone tests with all NR cells in FR1	1989

A.6.1	SA: RRC_IDLE state mobility	1989

A.6.1.1	Cell re-selection to NR	1989

A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case	1989

A.6.1.1.1.1	Test Purpose and Environment	1989

A.6.1.1.1.2	Test Parameters	1989

A.6.1.1.1.3	Test Requirements	1993

A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case	1993

A.6.1.1.2.1	Test Purpose and Environment	1993

A.6.1.1.2.2	Test Parameters	1993

A.6.1.1.2.3	Test Requirements	1997

A.6.1.1.3	Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1997

A.6.1.1.3.1	Test Purpose and Environment	1997

A.6.1.1.3.2	Test Parameters	1997

A.6.1.1.3.3	Test Requirements	2002

A.6.1.1.4	Cell reselection to FR1 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2002

A.6.1.1.4.1	Test Purpose and Environment	2002

A.6.1.1.4.2	Test Parameters	2002

A.6.1.1.4.3	Test Requirements	2005

A.6.1.1.5	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2005

A.6.1.1.5.1	Test Purpose and Environment	2005

A.6.1.1.5.2	Test Parameters	2005

A.6.1.1.5.3	Test Requirements	2010

A.6.1.1.6	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2010

A.6.1.1.6.1	Test Purpose and Environment	2010

A.6.1.1.6.2	Test Parameters	2011

A.6.1.1.6.3	Test Requirements	2015

A.6.1.1.7	Cell reselection to FR1 intra-frequency NR case for UE configured with highSpeedMeasFlag-r16	2016

A.6.1.1.7.1	Test Purpose and Environment	2016

A.6.1.1.7.2	Test Parameters	2016

A.6.1.1.7.3	Test Requirements	2019

A.6.1.1.8	Cell reselection to FR1 inter-frequency NR case for UE configured with highSpeedMeasInterFreq-r17	2019

A.6.1.1.8.1	Test Purpose and Environment	2019

A.6.1.1.8.2	Test Parameters	2019

A.6.1.1.8.3	Test Requirements	2023

A.6.1.2	Inter-RAT E-UTRAN cell re-selection	2023

A.6.1.2.1	Cell reselection to higher priority E-UTRAN	2023

A.6.1.2.1.1	Test Purpose and Environment	2023

A.6.1.2.1.2	Test Parameters	2023

A.6.1.2.1.3	Test Requirements	2026

A.6.1.2.2	Cell reselection to lower priority E-UTRAN	2027

A.6.1.2.2.1	Test Purpose and Environment	2027

A.6.1.2.2.2	Test Parameters	2027

A.6.1.2.2.3	Test Requirements	2030

A.6.1.2.3	Cell reselection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	2031

A.6.1.2.3.1	Test Purpose and Environment	2031

A.6.1.2.3.2	Test Parameters	2031

A.6.1.2.3.3	Test Requirements	2034

A.6.1.2.4	Cell reselection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	2035

A.6.1.2.4.1	Test Purpose and Environment	2035

A.6.1.2.4.2	Test Parameters	2035

A.6.1.2.4.3	Test Requirements	2038

A.6.1.2.5	Cell reselection to lower priority E-UTRAN cell for UE configured with highSpeedMeasFlag-r16	2039

A.6.1.2.5.1	Test Purpose and Environment	2039

A.6.1.2.5.2	Test Parameters	2039

A.6.1.2.5.3	Test Requirements	2043

A.6.1.1.7	Void	2044

A.6.2	SA: RRC_INACTIVE state mobility	2044

A.6.2.1	Configured Grant based Small Data Transmissions (CG-SDT)	2044

A.6.2.1.1	Test purpose and Environment	2044

A.6.2.1.2	Test Parameters	2045

A.6.2.1.3	Test requirements	2047

A.6.3	RRC_CONNECTED state mobility	2047

A.6.3.1	Handover	2047

A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell	2047

A.6.3.1.1.1	Test Purpose and Environment	2047

A.6.3.1.1.2	Test Parameters	2047

A.6.3.1.1.3 Test Requirements	2051

A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell	2051

A.6.3.1.2.1	Test Purpose and Environment	2051

A.6.3.1.2.2	Test Parameters	2051

A.6.3.1.2.3	Test Requirements	2055

A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell	2055

A.6.3.1.3.1	Test Purpose and Environment	2055

A.6.3.1.3.2	Test Parameters	2055

A.6.3.1.3.3	Test Requirements	2059

A.6.3.1.4	SA NR - E-UTRAN handover	2059

A.6.3.1.4.1	Test Purpose and Environment	2059

A.6.3.1.4.2	Test Requirements	2065

A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell	2065

A.6.3.1.5.1	Test Purpose and Environment	2065

A.6.3.1.5.2	Test Requirements	2071

A.6.3.1.6	 SA NR - UTRAN FDD handover	2071

A.6.3.1.6.1	Test Purpose and Environment	2071

A.6.3.1.6.2	Test Requirements	2075

A.6.3.1.7	Intra-frequency synchronous DAPS handover in FR1	2075

A.6.3.1.7.1	Test Purpose and Environment	2075

A.6.3.1.7.2	Test Parameters	2075

A.6.3.1.7.3	Test Requirements	2079

A.6.3.1.8	Intra-frequency asynchronous DAPS handover in FR1	2080

A.6.3.1.8.1	Test Purpose and Environment	2080

A.6.3.1.8.2	Test Parameters	2080

A.6.3.1.8.3	Test Requirements	2084

A.6.3.1.9	Intra-band inter-frequency synchronous DAPS handover test in SA for FR1	2085

A.6.3.1.9.1	Test Purpose and Environment	2085

A.6.3.1.9.2	Test Parameters	2085

A.6.3.1.9.3	Test Requirements	2089

A.6.3.1.10	Intra-band inter-frequency asynchronous DAPS handover test in SA for FR1	2089

A.6.3.1.10.1	Test Purpose and Environment	2089

A.6.3.1.10.2	Test Parameters	2089

A.6.3.1.10.3	Test Requirements	2092

A.6.3.1.11	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1	2092

A.6.3.1.11.1	Test Purpose and Environment	2092

A.6.3.1.11.2	Test Parameters	2092

A.6.3.1.11.3 Test Requirements	2099

A.6.3.1.12	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1	2099

A.6.3.1.12.1	Test Purpose and Environment	2099

A.6.3.1.12.2	Test Parameters	2099

A.6.3.1.12.3 Test Requirements	2107

A.6.3.1.13	SA NR - E-UTRAN with NR PSCell addition in FR1	2107

A.6.3.1.13.1	Test Purpose and Environment	2107

A.6.3.1.13.2	Test Requirements	2116

A.6.3.1.14	SA NR - E-UTRAN handover with NR FR1 PScell addition	2116

A.6.3.1.14.1	Test Purpose and Environment	2116

A.6.3.1.14.2	Test Requirements	2125

A.6.3.2	RRC Connection Mobility Control	2126

A.6.3.2.1	SA: RRC Re-establishment	2126

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1	2126

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1	2130

A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing	2134

A.6.3.2.2	Random Access	2137

A.6.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	2137

A.6.3.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	2141

A.6.3.2.2.3	2-step RA type contention based random access test in FR1 for NR standalone	2146

A.6.3.2.2.4	2-step RA type non-contention based test in FR1 for NR standalone	2151

A.6.3.2.3	SA: RRC Connection Release with Redirection	2155

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1	2155

A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN	2159

A.6.3.3 Conditional handover	2166

A.6.3.3.1	Intra-frequency conditional handover from FR1 to FR1	2166

A.6.3.3.1.1	Test Purpose and Environment	2166

A.6.3.3.1.2	Test Parameters	2166

A.6.3.3.1.3 Test Requirements	2170

A.6.3.3.2	Inter-frequency conditional handover from FR1 to FR1	2170

A.6.3.3.2.1	Test Purpose and Environment	2170

A.6.3.3.2.2	Test Parameters	2170

A.6.3.3.2.3 Test Requirements	2174

A.6.4	Timing	2174

A.6.4.1	UE transmit timing	2174

A.6.4.1.1	NR UE Transmit Timing Test for FR1	2174

A.6.4.1.1.1	Test Purpose and environment	2174

A.6.4.1.1.2	Test requirements	2178

A.6.4.2	UE timer accuracy	2178

A.6.4.3	Timing advance	2178

A.6.4.3.1	SA FR1 timing advance adjustment accuracy	2178

A.6.4.3.1.1	Test Purpose and Environment	2178

A.6.4.3.1.2	Test Parameters	2178

A.6.4.3.1.3	Test Requirements	2182

A.6.5	Signalling characteristics	2182

A.6.5.1	Radio link Monitoring	2182

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	2183

A.6.5.1.1.1	Test Purpose and Environment	2183

A.6.5.1.1.2	Test Requirements	2188

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	2188

A.6.5.1.2.1	Test Purpose and Environment	2188

A.6.5.1.2.2	Test Requirements	2194

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	2194

A.6.5.1.3.1	Test Purpose and Environment	2194

A.6.5.1.3.2	Test Requirements	2200

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	2200

A.6.5.1.4.1	Test Purpose and Environment	2200

A.6.5.1.4.2	Test Requirements	2206

A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	2206

A.6.5.1.5.1	Test Purpose and Environment	2206

A.6.5.1.5.2	Test Requirements	2212

A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	2212

A.6.5.1.6.1	Test Purpose and Environment	2212

A.6.5.1.6.2	Test Requirements	2217

A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	2217

A.6.5.1.7.1	Test Purpose and Environment	2217

A.6.5.1.7.2	Test Requirements	2222

A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	2222

A.6.5.1.8.1	Test Purpose and Environment	2222

A.6.5.1.8.2	Test Requirements	2227

A.6.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion	2227

A.6.5.1.9.1	Test Purpose and Environment	2227

A.6.5.1.9.2	Test Requirements	2233

A.6.5.2	Interruption	2233

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1	2233

A.6.5.2.1.2	Test Requirements	2237

A.6.5.2.2	SA interruptions at NR SRS carrier based switching	2238

A.6.5.2.2.1	Test Purpose and Environment	2238

A.6.5.2.2.2	Test Parameters	2238

A.6.5.2.2.3	Test Requirements	2240

A.6.5.2.3	SA interruptions at NR SRS antenna port switching with 1 SRS symbol in a slot in NR-CA	2241

A.6.5.2.3.1	Test Purpose and Environment	2241

A.6.5.2.3.2	Test Parameters	2241

A.6.5.2.3.3	Test Requirements	2243

A.6.5.2.4	SA interruptions at NR SRS antenna port switching	2244

A.6.5.2.4.1	Test Purpose and Environment	2244

A.6.5.2.4.2	Test Parameters	2244

A.6.5.2.4.3	Test Requirements	2246

A.6.5.3	SCell Activation and Deactivation Delay	2247

A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	2247

A.6.5.3.1.1	Test Purpose and Environment	2247

A.6.5.3.1.2	Test Requirements	2252

A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	2253

A.6.5.3.2.1	Test Purpose and Environment	2253

A.6.5.3.2.2	Test Requirements	2253

A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	2253

A.6.5.3.3.1	Test Purpose and Environment	2253

A.6.5.3.3.2	Test Requirements	2254

A.6.5.3.4	Direct SCell activation at SCell addition of known SCell in FR1	2254

A.6.5.3.4.1	Test Purpose and Environment	2254

A.6.5.3.4.2	Test Requirements	2258

A.6.5.3.5	Direct SCell activation at handover with known SCell in FR1	2258

A.6.5.3.5.1	Test Purpose and Environment	2258

A.6.5.3.5.2	Test Requirements	2263

A.6.5.3.4	Fast SCell Activation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	2263

A.6.5.3.4.1	Test Purpose and Environment	2263

A.6.5.3.4.2	Test Requirements	2267

A.6.5.3.5	SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	2267

A.6.5.3.5.1	Test Purpose and Environment	2267

A.6.5.3.5.2	Test Requirements	2268

A.6.5.3.6	PUCCH SCell Activation and deactivation of known SCell in FR1	2268

A.6.5.3.6.1	Test Purpose and Environment	2268

A.6.5.3.6.2	Test Requirements	2272

A.6.5.3.7	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	2272

A.6.5.3.7.1	Test Purpose and Environment	2272

A.6.5.3.7.2	Test Requirements	2276

A.6.5.3.8	SCell Activation and Deactivation of one FR1 known PUCCH SCell and one FR1 unknown SCell with single activation/deactivation command	2277

A.6.5.3.8.1	Test Purpose and Environment	2277

A.6.5.3.8.2	Test Requirements	2281

A.6.5.3.9	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR1 in non-DRX	2282

A.6.5.3.9.1	Test Purpose and Environment	2282

A.6.5.3.9.2	Test Requirements	2286

A.6.5.4	UE UL carrier RRC reconfiguration Delay	2287

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay	2287

A.6.5.4.1.1	Test Purpose and Environment	2287

A.6.5.4.1.2	Test Requirements	2297

A.6.5.4.2	Void	2298

A.6.5.5	Beam Failure Detection and Link recovery procedures	2298

A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	2298

A.6.5.5.1.1	Test Purpose and Environment	2298

A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	2303

A.6.5.5.2.1	Test Purpose and Environment	2303

A.6.5.5.2.2	Test Requirements	2310

A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	2310

A.6.5.5.3.1	Test Purpose and Environment	2310

A.6.5.5.3.2	Test Requirements	2316

A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	2316

A.6.5.5.4.1	Test Purpose and Environment	2316

A.6.5.5.4.2	Test Requirements	2322

A.6.5.5.5	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	2322

A.6.5.5.5.1	Test Purpose and Environment	2322

A.6.5.5.5.2	Test Requirements	2326

A.6.5.5.6	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	2327

A.6.5.5.6.1	Test Purpose and Environment	2327

A.6.5.5.6.2	Test Requirements	2331

A.6.5.5.7	TRP Specific Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	2331

A.6.5.5.7.1	Test Purpose and Environment	2331

A.6.5.5.7.2	Test Requirements	2337

A.6.5.6	Active BWP switch	2337

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch	2337

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of SCell with non-DRX in SA	2337

A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA	2343

A.6.5.6.2	RRC-based Active BWP Switch	2348

A.6.5.6.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	2348

A.6.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	2352

A.6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2352

A.6.5.6.3.1.1	Test Purpose and Environment	2352

A.6.5.6.3.1.2	Test Requirements	2359

A.6.5.6.4	SCell dormancy switch	2359

A.6.5.6.4.1	NR FR1 PCell SCell dormancy switch of single FR1 SCell outside active time	2359

A.6.5.6.4.1.1	Test Purpose and Environment	2359

A.6.5.6.4.1.2	Test Requirements	2365

A.6.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR1 SCells inside active time	2365

A.6.5.6.4.2.1	 Test Purpose and Environment	2365

A.6.5.6.4.2.2	Test Requirements	2372

A.6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	2372

A.6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2372

A.6.5.7	DL interruptions at switching between two uplink carriers	2377

A.6.5.7.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	2377

A.6.5.7.1.1	Test Purpose and Environment	2377

A.6.5.7.1.2	Test Requirements	2381

A.6.5.7.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	2381

A.6.5.7.2.1	Test Purpose and Environment	2381

A.6.5.7.2.2	Test Requirements	2385

A.6.5.7A	DL interruptions at switching between two uplink carriers with two transmit antenna connectors	2385

A.6.5.7A.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	2385

A.6.5.7A.1.1	Test Purpose and Environment	2385

A.6.5.7A.1.2	Test Requirements	2389

A.6.5.7A.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	2389

A.6.5.7A.2.1	Test Purpose and Environment	2389

A.6.5.7A.2.2	Test Requirements	2393

A.6.5.7B	DL interruptions at switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	2393

A.6.5.7B.1	DL interruptions at switching between two uplink bands in FDD-TDD CA	2393

A.6.5.7B.1.1	Test Purpose and Environment	2393

A.6.5.7B.1.2	Test Requirements	2397

A.6.5.7B.2	DL interruptions at switching between two uplink bands in TDD-TDD CA	2397

A.6.5.7B.2.1	Test Purpose and Environment	2397

A.6.5.7B.2.2	Test Requirements	2401

A.6.5.7C	DL interruptions at switching between two uplink bands with two transmit antenna connectors	2401

A.6.5.7C.1	DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA	2401

A.6.5.7C.1.1	Test Purpose and Environment	2401

A.6.5.7C.1.2	Test Requirements	2405

A.6.5.7C.2	DL interruptions at switching between two uplink bands with two transmit antenna connectors in TDD-TDD CA	2406

A.6.5.7C.2.1	Test Purpose and Environment	2406

A.6.5.7C.2.2	Test Requirements	2410

A.6.5.8	UE specific CBW change	2411

A.6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	2411

A.6.5.8.1.1	Test Purpose and Environment	2411

A.6.5.8.1.2	Test Requirements	2415

A.6.5.9	Pathloss reference signal switching delay	2415

A.6.5.9.1	MAC-CE based pathloss reference signal switch delay	2415

A.6.5.9.1.1	Test Purpose and Environment	2415

A.6.5.9.1.2	Test Requirements	2418

A.6.6	Measurement procedure	2419

A.6.6.1	Intra-frequency Measurements	2419

A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX	2419

A.6.6.1.1.1	Test purpose and Environment	2419

A.6.6.1.1.2	Test parameters	2419

A.6.6.1.1.3	Test Requirements	2423

A.6.6.1.2	SA event triggered reporting tests without gap under DRX	2423

A.6.6.1.2.1	Test purpose and Environment	2423

A.6.6.1.2.2	Test parameters	2423

A.6.6.1.2.3	Test Requirements	2427

A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX	2427

A.6.6.1.3.1	Test purpose and Environment	2427

A.6.6.1.3.2	Test parameters	2427

A.6.6.1.3.3	Test Requirements	2431

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX	2431

A.6.6.1.4.1	Test purpose and Environment	2431

A.6.6.1.4.2	Test parameters	2431

A.6.6.1.4.3	Test Requirements	2435

A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading	2435

A.6.6.1.5.1	Test purpose and Environment	2435

A.6.6.1.5.2	Test parameters	2435

A.6.6.1.5.3	Test Requirements	2437

A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	2438

A.6.6.1.6.1	Test purpose and Environment	2438

A.6.6.1.6.2	Test parameters	2438

A.6.6.1.6.3	Test Requirements	2439

A.6.6.1.7	SA event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	2440

A.6.6.1.7.1	Test purpose and Environment	2440

A.6.6.1.7.2	Test parameters	2440

A.6.6.1.7.3	Test Requirements	2444

A.6.6.1.8	SA event triggered reporting tests without gap under DRX for UE configured with highSpeedMeasCA-Scell-r17	2444

A.6.6.1.8.1	Test purpose and Environment	2444

A.6.6.1.8.2	Test parameters	2444

A.6.6.1.8.3	Test Requirements	2448

A.6.6.2	Inter-frequency Measurements	2448

A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2448

A.6.6.2.1.1	Test Purpose and Environment	2448

A.6.6.2.1.2	Test Requirements	2452

A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2452

A.6.6.2.2.1	Test Purpose and Environment	2452

A.6.6.2.2.2	Test Requirements	2456

A.6.6.2.3	Void	2457

A.6.6.2.4	Void	2457

A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2457

A.6.6.2.5.1	Test Purpose and Environment	2457

A.6.6.2.5.2	Test Requirements	2461

A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2461

A.6.6.2.6.1	Test Purpose and Environment	2461

A.6.6.2.6.2	Test Requirements	2465

A.6.6.2.7	Void	2466

A.6.6.2.8	Void	2466

A.6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern	2466

A.6.6.2.9.1	Test Purpose and Environment	2466

A.6.6.2.9.2	Test Requirements	2469

A.6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used	2469

A.6.6.2.10.1	Test Purpose and Environment	2469

A.6.6.2.10.2	Test Requirements	2473

A.6.6.2.12	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for UE configured with highSpeedMeasInterFreq-r17	2477

A.6.6.2.12.1	Test Purpose and Environment	2477

A.6.6.2.12.2	Test Requirements	2481

A.6.6.3	Inter-RAT Measurements	2481

A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	2481

A.6.6.3.1.1	Test Purpose and Environment	2481

A.6.6.3.1.2	Test Requirements	2487

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	2487

A.6.6.3.2.1	Test Purpose and Environment	2487

A.6.6.3.2.2	Test Requirements	2494

A.6.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for UE configured with highSpeedMeasFlag-r16	2494

A.6.6.3.3.1	Test Purpose and Environment	2494

A.6.6.3.3.2	Test Requirements	2499

A.6.6.4	L1-RSRP measurement for beam reporting	2499

A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used	2499

A.6.6.4.1.1	Test Purpose and Environment	2499

A.6.6.4.1.2	Test parameters	2500

A.6.6.4.1.3	Test Requirements	2503

A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used	2503

A.6.6.4.2.1	Test Purpose and Environment	2503

A.6.6.4.2.2	Test parameters	2504

A.6.6.4.2.3	Test Requirements	2507

A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	2507

A.6.6.4.3.1	Test Purpose and Environment	2507

A.6.6.4.3.2	Test parameters	2508

A.6.6.4.3.3	Test Requirements	2511

A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	2511

A.6.6.4.4.1	Test Purpose and Environment	2511

A.6.6.4.4.2	Test parameters	2512

A.6.6.4.4.3	Test Requirements	2515

A.6.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	2515

A.6.6.4.5.1	Test Purpose and Environment	2515

A.6.6.4.5.2	Test parameters	2516

A.6.6.4.5.3	Test Requirements	2518

A.6.6.4.6	Inter-cell SSB based L1-RSRP measurements on FR1 PCell when DRX is used	2518

A.6.6.4.6.1	Test Purpose and Environment	2518

A.6.6.4.6.2	Test parameters	2518

A.6.6.4.6.3	Test Requirements	2521

A.6.6.5	Inter-RAT UTRAN FDD measurements	2522

A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1	2522

A.6.6.5.1.1	Test Purpose and Environment	2522

A.6.6.5.1.2	Test Requirements	2525

A.6.6.6	CLI measurements	2525

A.6.6.6.1	SRS-RSRP measurement with DRX	2525

A.6.6.6.1.1	Test Purpose and Environment	2525

A.6.6.6.1.2	Test Parameters	2526

A.6.6.6.1.3	Test Requirements	2529

A.6.6.6.2	CLI-RSSI measurement with DRX	2529

A.6.6.6.2.1	Test Purpose and Environment	2529

A.6.6.6.2.2	Test Parameters	2530

A.6.6.6.2.3	Test Requirements	2532

A.6.6.7	NR measurements with autonomous gaps	2532

A.6.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	2532

A.6.6.7.1.1	Test Purpose and Environment	2532

A.6.6.7.1.2	Test Parameters	2532

A.6.6.7.1.3	Test Requirements	2535

A.6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	2535

A.6.6.7.2.1	Test Purpose and Environment	2535

A.6.6.7.2.2	Test Requirements	2538

A.6.6.8	L1-SINR measurement for beam reporting	2539

A.6.6.8.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	2541

A.6.6.8.2.1	Test Purpose and Environment	2541

A.6.6.8.2.2	Test parameters	2541

A.6.6.8.2.3	Test Requirements	2545

A.6.6.8.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	2545

A.6.6.8.3.1	Test Purpose and Environment	2545

A.6.6.8.3.2	Test parameters	2546

A.6.6.8.3.3	Test Requirements	2548

A.6.6.9	Idle Mode CA/DC Measurements	2548

A.6.6.9.1	SA Idle mode CA/DC measurement for FR1	2548

A.6.6.9.1.1	Test Purpose and Environment	2548

A.6.6.9.1.2	Test Requirements	2555

A.6.6.10	CSI-RS based intra-frequency Measurements	2555

A.6.6.10.1	SA event triggered reporting tests without gap under non-DRX	2555

A.6.6.10.1.1	Test purpose and Environment	2555

A.6.6.10.1.2	Test Requirements	2558

A.6.6.11	CSI-RS based inter-frequency Measurements	2558

A.6.6.11.1	 SA event triggered reporting tests with gap under DRX	2558

A.6.6.11.1.1	Test Purpose and Environment	2558

A.6.6.11.1.2	Test Requirements	2562

A.6.6.12	RSTD measurements	2562

A.6.6.12.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA	2562

A.6.6.12.1.1	Test Purpose and Environment	2562

A.6.6.12.1.2	Test Requirements	2569

A.6.6.12.2	NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR1 SA	2570

A.6.6.12.2.1	Test Purpose and Environment	2570

A.6.6.12.2.2	Test Requirements	2576

A.6.6.12.3	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA	2576

A.6.6.12.3.1	Test Purpose and Environment	2576

A.6.6.12.3.2	Test Requirements	2582

A.6.6.12.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA without measurement gap	2582

A.6.6.12.4.1	Test Purpose and Environment	2582

A.6.6.12.4.2	Test Requirements	2586

A.6.6.12.5	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_CONNECTED state with Rx TEG	2587

A.6.6.12.5.1	Test Purpose and Environment	2587

A.6.6.12.5.2	Test Requirements	2591

A.6.6.13 PRS-RSRP measurements	2591

A.6.6.13.1	PRS-RSRP reporting delay test case for single positioning frequency layer	2591

A.6.6.13.1.1	Test purpose and Environment	2591

A.6.6.13.1.2	Test Requirements	2595

A.6.6.13.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	2595

A.6.6.13.2.1	Test purpose and Environment	2595

A.6.6.13.2.2	Test Requirements	2599

A.6.6.13.3	PRS-RSRP reporting delay test case for reduced number of samples	2599

A.6.6.13.3.1	Test purpose and Environment	2599

A.6.6.13.3.2	Test Requirements	2603

A.6.6.13.4	PRS-RSRP reporting delay test case for single positioning frequency layer outside MG	2603

A.6.6.13.4.1	Test purpose and Environment	2603

A.6.6.14	UE Rx-Tx time difference measurements	2607

A.6.6.14.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	2607

A.6.6.14.1.1	Test purpose and environment	2607

A.6.6.14.1.2	Test requirements	2611

A.6.6.14.2	UE Rx-Tx time difference measurement for dual positioning frequency layers in FR1 SA	2611

A.6.6.14.2.1	Test purpose and environment	2611

A.6.6.14.2.2	Test requirements	2615

A.6.6.14.3	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with reduced sample number	2615

A.6.6.14.3.1	Test purpose and environment	2615

A.6.6.14.3.2	Test requirements	2619

A.6.6.14.4	UE Rx-Tx time difference measurement without gaps in FR1 SA	2619

A.6.6.14.4.1	Test purpose and environment	2619

A.6.6.14.4.2	Test requirements	2623

A.6.6.14.5	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with multiple RxTx TEGs	2623

A.6.6.14.4.1	Test purpose and environment	2623

A.6.6.14.4.2	Test requirements	2627

A.6.6.15	Idle Mode measurements of inter-RAT CA candidate cells for early reporting	2627

A.6.6.15.1	Test Purpose and Environment	2627

A.6.6.15.2	Test Requirements	2634

A.6.6.16	PRS-RSRPP measurements	2635

A.6.6.16.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state	2635

A.6.6.16.1.1	Test purpose and Environment	2635

A.6.6.16.1.2	Test Requirements	2637

A.6.6.16.2	PRS-RSRPP reporting delay test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state	2637

A.6.6.16.2.1	Test purpose and Environment	2637

A.6.6.16.2.2	Test Requirements	2640

A.6.6.16.3	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state without measurement gap	2640

A.6.6.16.3.1	Test purpose and Environment	2640

A.6.6.16.3.2	Test Requirements	2644

A.6.6.17	SA event triggered reporting tests with Pre-MG	2644

A.6.6.17.1	SA event triggered reporting tests with autonomous activation/deactivation Pre-MG	2644

A.6.6.17.1.1	Test purpose and Environment	2644

A.6.6.17.1.2	Test parameters	2644

A.6.6.17.1.3	Test Requirements	2648

A.6.6.17.2	SA event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation	2648

A.6.6.17.2.1	Test purpose and Environment	2648

A.6.6.17.2.2	Test parameters	2648

A.6.6.17.2.3	Test Requirements	2653

A.6.6.17.3	Void	2653

A.6.6.17.3.1	Void	2653

A.6.6.17.3.2	Void	2653

A.6.6.17.3.3	Void	2653

A.6.6.18	SA event triggered reporting tests with concurrent gaps	2653

A.6.6.18.1	SA event triggered reporting tests for FR1 concurrent gaps with non-overalpping scenario for SSB-based measurements in both inter-frequency layers	2653

A.6.6.18.1.1	Test Purpose and Environment	2653

A.6.6.18.1.2	Test Requirements	2657

A.6.6.18.2	SA event triggered reporting tests for FR1 concurrent gap with partially partial overalpping scenario for SSB-based measurements in both inter-frequency layers	2657

A.6.6.18.2.1	Test Purpose and Environment	2657

A.6.6.18.2.2	Test Requirements	2661

A.6.6.18.3	SA NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1	2661

A.6.6.18.3.1	Test Purpose and Environment	2661

A.6.6.18.3.2	Test Requirements	2668

A.6.6.18.4	SA event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used	2668

A.6.6.18.4.1	Test Purpose and Environment	2668

A.6.6.18.4.2	Test Requirements	2672

A.6.6.19	SA event triggered reporting tests with NCSG	2672

A.6.6.19.1	SA event triggered reporting tests with NCSG under non-DRX in FR1	2672

A.6.6.19.1.1	Test purpose and Environment	2672

A.6.6.19.1.2	Test parameters	2673

A.6.6.19.1.3	Test Requirements	2677

A.6.6.19.2	SA event triggered reporting tests for FR1 with NCSG for inter-frequency measurement	2677

A.6.6.19.2.1	Test Purpose and Environment	2677

A.6.6.19.2.2	Test parameters	2677

A.6.6.19.2.3	Test Requirements	2681

A.6.6.19.3	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG	2681

A.6.6.19.3.1	Test Purpose and Environment	2681

A.6.6.19.3.2	Test parameters	2682

A.6.6.19.3.3	Test Requirements	2687

A.6.6.19.4	Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX	2687

A.6.6.19.4.1	Test purpose and Environment	2687

A.6.6.19.4.2	Test parameters	2687

A.6.6.19.4.3	Test Requirements	2690

A.6.6.20	UE Rx-Tx time difference measurement for propagation delay compensation	2690

A.6.6.20.1	Test purpose and environment	2690

A.6.6.20.2	Test requirements	2694

A.6.6.21	UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA	2694

A.6.6.21.1	Test purpose and environment	2694

A.6.6.21.2	Test requirements	2696

A.6.7	Measurement Performance requirements	2697

A.6.7.1	SS-RSRP	2697

A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2697

A.6.7.1.1.1	Test Purpose and Environment	2697

A.6.7.1.1.2	Test parameters	2697

A.6.7.1.1.3	Test Requirements	2702

A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2702

A.6.7.1.2.1	Test Purpose and Environment	2702

A.6.7.1.2.2	Test parameters	2702

A.6.7.1.2.3	Test Requirements	2706

A.6.7.1.3	Void	2706

A.6.7.2	SS-RSRQ	2706

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2706

A.6.7.2.1.1	Test Purpose and Environment	2706

A.6.7.2.1.2	Test Parameters	2707

A.6.7.2.1.3	Test Requirements	2712

A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2712

A.6.7.2.2.1	Test Purpose and Environment	2712

A.6.7.2.2.2	Test Parameters	2712

A.6.7.2.2.3	Test Requirements	2717

A.6.7.3	SS-SINR	2717

A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2717

A.6.7.3.1.1	Test Purpose and Environment	2717

A.6.7.3.1.2	Test Parameters	2718

A.6.7.3.1.3	Test Requirements	2722

A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2722

A.6.7.3.2.1	Test Purpose and Environment	2722

A.6.7.3.2.2	Test Parameters	2723

A.6.7.3.2.3	Test Requirements	2727

A.6.7.4	L1-RSRP measurement for beam reporting	2728

A.6.7.4.1	SSB based L1-RSRP measurement	2728

A.6.7.4.1.1	Test Purpose and Environment	2728

A.6.7.4.1.2	Test parameters	2728

A.6.7.4.1.3	Test Requirements	2732

A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2732

A.6.7.4.2.1	Test Purpose and Environment	2732

A.6.7.4.2.2	Test parameters	2733

A.6.7.4.2.3	Test Requirements	2737

A.6.7.5	E-UTRAN RSRP	2737

A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2737

A.6.7.5.1.1	Test Purpose and Environment	2737

A.6.7.5.1.2	Test parameters	2738

A.6.7.5.1.3	Test Requirements	2744

A.6.7.6	E-UTRAN RSRQ	2745

A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2745

A.6.7.6.1.1	Test Purpose and Environment	2745

A.6.7.6.1.2	Test parameters	2745

A.6.7.6.1.3	Test Requirements	2750

A.6.7.7	E-UTRAN RS-SINR	2751

A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2751

A.6.7.7.1.1	Test Purpose and Environment	2751

A.6.7.7.1.2	Test parameters	2751

A.6.7.7.1.3	Test Requirements	2757

A.6.7.8	CLI measurements	2758

A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell	2758

A.6.7.8.1.1	Test Purpose and Environment	2758

A.6.7.8.1.2	Test parameters	2758

A.6.7.8.1.3	Test Requirements	2764

A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell	2764

A.6.7.8.2.1	Test Purpose and Environment	2764

A.6.7.8.2.2	Test parameters	2765

A.6.7.8.2.3	Test Requirements	2768

A.6.7.9	L1-SINR measurement for beam reporting	2769

A.6.7.9.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2773

A.6.7.9.2.1	Test Purpose and Environment	2773

A.6.7.9.2.2	Test parameters	2774

A.6.7.9.2.3	Test Requirements	2779

A.6.7.9.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2779

A.6.7.9.3.1	Test Purpose and Environment	2779

A.6.7.9.3.2	Test parameters	2779

A.6.7.9.3.3	Test Requirements	2784

A.6.7.10	CSI-RSRP	2784

A.6.7.10.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2784

A.6.7.9.10.1	Test Purpose and Environment	2784

A.6.7.9.10.2	Test parameters	2785

A.6.7.10.1.3	Test Requirements	2790

A.6.7.10.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2790

A.6.7.9.10.1	Test Purpose and Environment	2790

A.6.7.10.2.2	Test parameters	2790

A.6.7.10.2.3	Test Requirements	2795

A.6.7.11	CSI-RSRQ	2795

A.6.7.11.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2795

A.6.7.11.1.1	Test Purpose and Environment	2795

A.6.7.11.1.2	Test Parameters	2795

A.6.7.11.1.3	Test Requirements	2800

A.6.7.11.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2800

A.6.7.11.2.1	Test Purpose and Environment	2800

A.6.7.11.2.2	Test Parameters	2800

A.6.7.11.2.3	Test Requirements	2806

A.6.7.12	CSI-SINR	2807

A.6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2807

A.6.7.12.1.1	Test Purpose and Environment	2807

A.6.7.12.1.2	Test Parameters	2807

A.6.7.12.1.3	Test Requirements	2811

A.6.7.12.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2812

A.6.7.12.2.1	Test Purpose and Environment	2812

A.6.7.12.2.2	Test Parameters	2812

A.6.7.12.2.3	Test Requirements	2818

A.6.7.13	RSTD measurements	2818

A.6.7.13.1	RSTD measurement accuracy test case for single positioning frequency layer	2818

A.6.7.13.1.1	Test purpose and Environment	2818

A.6.7.13.1.2	Test Requirements	2821

A.6.7.13.2	RSTD measurement accuracy test case for dual positioning frequency layer	2821

A.6.7.13.2.1	Test purpose and Environment	2821

A.6.7.13.2.2	Test Requirements	2825

A.6.7.13.3	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state	2825

A.6.7.13.3.1	Test purpose and Environment	2825

A.6.7.13.3.2	Test Requirements	2827

A.6.7.13.4	RSTD measurement accuracy test case with Rx TEG	2827

A.6.7.14	PRS-RSRP measurements	2831

A.6.7.14.1	SA: measurement accuracy with PRS in FR1	2831

A.6.7.14.1.1	Test Purpose and Environment	2831

A.6.7.14.1.2	Test parameters	2832

A.6.7.14.1.3	Test Requirements	2836

A.6.7.14.2	SA: measurement accuracy with PRS in FR1 with reduced sample number	2836

A.6.7.14.2.1	Test Purpose and Environment	2836

A.6.7.14.2.2	Test parameters	2836

A.6.7.14.2.3	Test Requirements	2839

A.6.7.14.3	Void	2840

A.6.7.14.3.1	Void	2840

A.6.7.14.3.2	Void	2840

A.6.7.14.3.3	Void	2840

A.6.7.15	UE Rx-Tx time difference measurements	2840

A.6.7.15.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA	2840

A.6.7.15.1.1	Test purpose and environment	2840

A.6.7.15.1.2	Test parameters	2840

A.6.7.15.1.3	Test requirements	2843

A.6.7.15.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR1 SA	2844

A.6.7.15.2.1	Test purpose and environment	2844

A.6.7.15.2.2	Test parameters	2844

A.6.7.15.2.3	Test requirements	2847

A.6.7.15.3	UE Rx-Tx time difference measurement accuracy with RxTx TEG	2847

A.6.7.15.3.1	Test purpose and environment	2847

A.6.7.15.3.2	Test parameters	2848

A.6.7.15.3.3	Test requirements	2851

A.6.7.16	PRS-RSRPP measurements	2851

A.6.7.16.1	SA: measurement accuracy with PRS in FR1	2851

A.6.7.16.1.1	Test Purpose and Environment	2851

A.6.7.16.1.2	Test parameters	2852

A.6.7.16.1.3	Test Requirements	2857

A.6.7.16.2	SA: measurement accuracy with reduced PRS samples in FR1	2857

A.6.7.16.2.1	Test Purpose and Environment	2857

A.6.7.16.2.2	Test parameters	2857

A.6.8	Measurement procedure in RRC_INACTIVE	2861

A.6.8.1	RSTD measurements	2861

A.6.8.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_INACTIVE state	2861

A.6.8.1.1.1	Test Purpose and Environment	2861

A.6.8.1.1.2	Test Requirements	2864

A.6.8.1.2	 NR RSTD measurement reporting delay test case with reduced number of samples in RRC_INACTIVE, FR1 SA	2864

A.6.8.1.2.1	Test Purpose and Environment	2864

A.6.8.1.2.2	Test Requirements	2869

A.6.8.2	PRS-RSRP measurements	2869

A.6.8.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	2869

A.6.8.2.1.1	Test purpose and Environment	2869

A.6.8.2.1.2	Test Requirements	2871

A.6.8.2.2	PRS-RSRP reporting delay test case with reduced number of samples in RRC_INACTIVE	2872

A.6.8.2.2.1	Test purpose and Environment	2872

A.6.8.2.2.2	Test Requirements	2875

A.6.8.3	UE Rx-Tx time difference measurements	2875

A.6.8.3.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	2875

A.6.8.3.1.1	Test purpose and environment	2875

A.6.8.3.1.2	Test requirements	2879

A.6.8.3.2	UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE, FR1 SA	2879

A.6.8.3.2.1	Test purpose and environment	2879

A.6.8.3.2.2	Test requirements	2882

A.6.8.4	PRS-RSRPP measurements	2882

A.6.8.4.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state	2882

A.6.8.4.1.1	Test purpose and Environment	2882

A.6.8.4.1.2	Test Requirements	2885

A.6.8.4.2	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state for reduced number of samples	2885

A.6.8.4.2.1	Test purpose and Environment	2885

A.6.8.4.2.2	Test Requirements	2887

A.6.9	Measurement performance requirements in RRC_INACTIVE	2887

A.6.9.1	RSTD measurements	2887

A.6.9.1.1	RSTD measurement accuracy test case for single positioning frequency layer in FR1 in RRC_INACTIVE state	2887

A.6.9.1.1.1	Test purpose and Environment	2887

A.6.9.1.1.2	Test Requirements	2889

A.6.9.1.2	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_INACTIVE state	2889

A.6.9.1.2.1	Test purpose and Environment	2889

A.6.9.1.2.2	Test Requirements	2891

A.6.9.2	PRS-RSRP measurements	2891

A.6.9.2.1	SA: measurement accuracy with PRS in FR1 in RRC_INACTIVE	2891

A.6.9.2.1.1	Test Purpose and Environment	2891

A.6.9.2.1.2	Test parameters	2892

A.6.9.2.1.3	Test Requirements	2897

A.6.9.2.2	SA: measurement accuracy with PRS in FR1 with reduced number of samples in RRC_INACTIVE state	2897

A.6.9.2.2.1	Test Purpose and Environment	2897

A.6.9.2.2.2	Test parameters	2897

A.6.9.2.2.3	Test Requirements	2900

A.6.9.3	UE Rx-Tx time difference measurements	2901

A.6.9.3.1.1	UE Rx-Tx time difference measurement accuracy in FR1 SA	2901

A.6.9.3.1.1.1	Test purpose and environment	2901

A.6.9.3.1.1.2 Test parameters	2901

A.6.9.3.1.1.3	Test requirements	2904

A.6.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples	2904

A.6.9.3.2.1	Test purpose and environment	2904

A.6.9.3.2.2	Test parameters	2904

A.6.9.3.2.3	Test requirements	2907

A.6.9.4	PRS-RSRPP measurements	2907

A.6.9.4.1	SA: PRS-RSRPP measurement accuracy in FR1 in RRC INACTIVE	2907

A.6.9.4.1.1	Test Purpose and Environment	2907

A.6.9.4.1.2	Test parameters	2907

A.6.9.4.1.3	Test Requirements	2912

A.6.9.4.2	SA: measurement accuracy with reduced PRS samples in FR1 in RRC INACTIVE	2912

A.6.9.4.2.1	Test Purpose and Environment	2912

A.6.9.4.2.2	Test parameters	2912

A.6.9.4.2.3	Test Requirements	2915

A.7	NR standalone tests with one or more NR cells in FR2	2949

A.7.1	SA: RRC_IDLE state mobility	2949

A.7.1.1	Cell re-selection to NR	2949

A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case	2949

A.7.1.1.1.1	Test Purpose and Environment	2949

A.7.1.1.1.2	Test Parameters	2949

A.7.1.1.1.3	Test Requirements	2952

A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case	2952

A.7.1.1.2.1	Test Purpose and Environment	2952

A.7.1.1.2.2	Test Parameters	2953

A.7.1.1.2.3	Test Requirements	2955

A.7.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2955

A.7.1.1.3.1	Test Purpose and Environment	2955

A.7.1.1.3.2	Test Parameters	2955

A.7.1.1.3.3	Test Requirements	2958

A.7.1.1.4	Cell reselection to FR2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2958

A.7.1.1.4.1	Test Purpose and Environment	2958

A.7.1.1.4.2	Test Parameters	2958

A.7.1.1.4.3	Test Requirements	2961

A.7.1.1.5	Cell reselection to FR2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2961

A.7.1.1.5.1	Test Purpose and Environment	2961

A.7.1.1.5.2	Test Parameters	2961

A.7.1.1.5.3	Test Requirements	2964

A.7.1.1.6	Cell reselection to FR2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2964

A.7.1.1.6.1	Test Purpose and Environment	2964

A.7.1.1.6.2	Test Parameters	2964

A.7.1.1.6.3	Test Requirements	2967

A.7.1.1.7	Cell reselection to FR2 intra-frequency NR case for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17	2967

A.7.1.1.7.1	Test Purpose and Environment	2967

A.7.1.1.7.2	Test Parameters	2967

A.7.1.1.7.3	Test Requirements	2971

A.7.2	SA: RRC_INACTIVE state mobility	2971

A.7.2.1	Small Data Transmission	2971

A.7.2.1.1	TA validation for CG-SDT in FR2	2971

A.7.2.1.1.1	Test Purpose and Environment	2971

A.7.2.1.1.2	Test Requirements	2974

A.7.3	RRC_CONNECTED state mobility	2975

A.7.3.1	Handover	2975

A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell	2975

A.7.3.1.1.1	Test Purpose and Environment	2975

A.7.3.1.1.2	Test Parameters	2975

A.7.3.1.1.3	Test Requirements	2978

A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell	2978

A.7.3.1.2.1	Test Purpose and Environment	2978

A.7.3.1.2.2	Test Parameters	2978

A.7.3.1.2.3	Test Requirements	2981

A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell	2981

A.7.3.1.3.1	Test Purpose and Environment	2981

A.7.3.1.3.2	Test Parameters	2981

A.7.3.1.3.3	Test Requirements	2982

A.7.3.1.4	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2	2983

A.7.3.1.4.1	Test Purpose and Environment	2983

A.7.3.1.4.2	Test Parameters	2983

A.7.3.1.4.3 Test Requirements	2990

A.7.3.1.5	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2	2990

A.7.3.1.5.1	Test Purpose and Environment	2990

A.7.3.1.5.2	Test Parameters	2990

A.7.3.1.5.3 Test Requirements	2997

A.7.3.1.6	Handover with PSCell from SA to EN-DC; unknown FR2 target cell	2997

A.7.3.1.6.1	Test Purpose and Environment	2997

A.7.3.1.6.2	Test Parameters	2997

A.7.3.1.6.3	Test Requirements	3002

A.7.3.1.7	HO with PSCell from FR1 NR-SA to EN-DC with known E-UTRA PCell and known FR2 PSCell	3002

A.7.3.1.7.1	Test purpose and environment	3002

A.7.3.1.7.2	Test Requirements	3008

A.7.3.1.8	NR PSCell change delay in HO with PSCell from NR-DC to NR-DC	3008

A.7.3.1.8.1	Test Purpose and Environment	3008

A.7.3.1.8.2	Test Requirements	3014

A.7.3.1.9	Intra-frequency handover from FR2-2 to FR2-2; unknown target cell	3014

A.7.3.1.9.1	Test Purpose and Environment	3014

A.7.3.1.9.2	Test Parameters	3014

A.7.3.1.9.3	Test Requirements	3017

A.7.3.1.10	Inter-frequency handover from FR2-2 to FR2-2; unknown target cell	3017

A.7.3.1.10.1	Test Purpose and Environment	3017

A.7.3.1.10.2	Test Parameters	3017

A.7.3.1.10.3	Test Requirements	3021

A.7.3.1.11	Inter-frequency handover from FR1 to FR2-2; unknown target cell	3021

A.7.3.1.11.1	Test Purpose and Environment	3021

A.7.3.1.11.2	Test Parameters	3021

A.7.3.1.11.3	Test Requirements	3025

A.7.3.2	RRC Connection Mobility Control	3025

A.7.3.2.1	SA: RRC Re-establishment	3025

A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	3025

A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	3028

A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	3031

A.7.3.2.1.3.1	Test Purpose and Environment	3031

A.7.3.2.1.3.2	Test Requirements	3033

A.7.3.2.1.4	Intra-frequency RRC Re-establishment in FR2-2	3034

A.7.3.2.1.4.1	Test Purpose and Environment	3034

A.7.3.2.1.4.2	Test Requirements	3036

A.7.3.2.1.5	Inter-frequency RRC Re-establishment in FR2-2	3037

A.7.3.2.1.5.1	Test Purpose and Environment	3037

A.7.3.2.1.5.2	Test Requirements	3040

A.7.3.2.1.6	Intra-frequency RRC Re-establishment in FR2-2 without serving cell timing	3040

A.7.3.2.1.6.1	Test Purpose and Environment	3040

A.7.3.2.1.6.2	Test Requirements	3042

A.7.3.2.2	Random Access	3043

A.7.3.2.2.1	4-step RA type c ontention based random access test in FR2 for NR Standalone	3043

A.7.3.2.2.2	4-step RA type n on-contention based random access test in FR2 for NR Standalone	3047

A.7.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	3051

A.7.3.2.2.4	2-step RA type n on-contention based random access test in FR2 for NR Standalone	3054

A.7.3.2.3	SA: RRC Connection Release with Redirection	3057

A.7.3.2.3.1	Redirection from NR in FR2 to NR in FR2	3057

A.7.3.3 Conditional Handover	3061

A.7.3.3.1	Intra-frequency conditional handover from FR2 to FR2	3061

A.7.3.3.1.1	Test Purpose and Environment	3061

A.7.3.3.1.2	Test Parameters	3061

A.7.3.3.1.2.3	Test Requirements	3064

A.7.3.3.2	Inter-frequency conditional handover from FR2 to FR2; unknown target cell	3064

A.7.3.3.2.1	Test Purpose and Environment	3064

A.7.3.3.2.2	Test Parameters	3064

A.7.3.3.2.3 Test Requirements	3066

A.7.4	Timing	3066

A.7.4.1	UE transmit timing	3066

A.7.4.1.1	NR UE Transmit Timing Test for FR2	3066

A.7.4.1.1.1	Test Purpose and environment	3066

A.7.4.1.1.2	Test requirements	3069

A.7.4.1.2	NR UE Transmit Timing Test for FR2-2	3070

A.7.4.1.2.1	Test Purpose and environment	3070

A.7.4.1.2.2	Test requirements	3074

A.7.4.2	UE timer accuracy	3075

A.7.4.3	Timing advance	3075

A.7.4.3.1	SA FR2 timing advance adjustment accuracy	3075

A.7.4.3.1.1	Test Purpose and Environment	3075

A.7.4.3.1.2	Test Parameters	3075

A.7.4.3.1.3 Test Requirements	3078

A.7.4.3.2	SA FR2-2 timing advance adjustment accuracy	3079

A.7.4.3.2.1	Test Purpose and Environment	3079

A.7.4.3.2.2	Test Parameters	3079

A.7.4.3.2.3	Test Requirements	3083

A.7.5	Signaling characteristics	3083

A.7.5.1	Radio link Monitoring	3083

A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3083

A.7.5.1.1.1	Test Purpose and Environment	3083

A.7.5.1.1.2	Test Requirements	3086

A.7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3087

A.7.5.1.2.1	Test Purpose and Environment	3087

A.7.5.1.2.2	Test Requirements	3091

A.7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	3092

A.7.5.1.3.1	Test Purpose and Environment	3092

A.7.5.1.3.2	Test Requirements	3096

A.7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	3096

A.7.5.1.4.1	Test Purpose and Environment	3096

A.7.5.1.4.2	Test Requirements	3101

A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	3101

A.7.5.1.5.1	Test Purpose and Environment	3101

A.7.5.1.5.2	Test Requirements	3106

A.7.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	3106

A.7.5.1.6.1	Test Purpose and Environment	3106

A.7.5.1.6.2	Test Requirements	3110

A.7.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	3110

A.7.5.1.7.1	Test Purpose and Environment	3110

A.7.5.1.7.2	Test Requirements	3114

A.7.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	3114

A.7.5.1.8.1	Test Purpose and Environment	3114

A.7.5.1.8.2	Test Requirements	3119

A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	3119

A.7.5.1.9.1	Test Purpose and Environment	3119

A.7.5.1.9.2	Test Requirements	3122

A.7.5.2	Interruption	3122

A.7.5.2.1	Interruptions during measurements on deactivated NR SCC in FR2	3122

A.7.5.2.1.1	Test Purpose and Environment	3122

A.7.5.2.1.2	Test Requirements	3125

A.7.5.2.2	SA interruptions at NR SRS carrier-based switching	3126

A.7.5.2.2.1	Test Purpose and Environment	3126

A.7.5.2.2.2	Test Parameters	3126

A.7.5.2.2.3	Test Requirements	3128

A.7.5.3	SCell Activation and Deactivation Delay	3128

A.7.5.3.1	SCell Activation and deactivation for SCell in FR2 intra-band in non-DRX	3128

A.7.5.3.1.1	Test Purpose and Environment	3128

A.7.5.3.1.2	Test Requirements	3130

A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2	3130

A.7.5.3.2.1	Test Purpose and Environment	3130

A.7.5.3.2.2	Test Requirements	3134

A.7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	3135

A.7.5.3.3.1	Test Purpose and Environment	3135

A.7.5.3.3.2	Test Requirements	3138

A.7.5.3.4	Direct SCell activation at SCell addition of known SCell in FR2	3139

A.7.5.3.4.1	Test Purpose and Environment	3139

A.7.5.3.4.2	Test Requirements	3142

A.7.5.3.5	Direct SCell activation at handover with known SCell in FR2	3143

A.7.5.3.5.1	Test Purpose and Environment	3143

A.7.5.3.5.2	Test Requirements	3145

A.7.5.3.6	PUCCH SCell activation and deactivation for FR1+FR2 inter-band with target SCell in FR2 and known	3146

A.7.5.3.6.1	Test Purpose and Environment	3146

A.7.5.3.6.2	Test Requirements	3150

A.7.5.3.7	PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR1 PCell	3151

A.7.5.3.7.1	Test Purpose and Environment	3151

A.7.5.3.7.2	Test Requirements	3155

A.7.5.3.8	SCell Activation and deactivation for known PUCCH SCell in FR2 inter-band in non-DRX	3156

A.7.5.3.8.1	Test Purpose and Environment	3156

A.7.5.3.8.2	Test Requirements	3160

A.7.5.3.9	PUCCH SCell Activation and deactivation of unknown SCell in FR2	3161

A.7.5.3.9.1	Test Purpose and Environment	3161

A.7.5.3.9.2	Test Requirements	3164

A.7.5.3.10	SCell Activation and deactivation of FR2 known PUCCH SCell and one FR2 unknown SCell with FR2 PCell	3165

A.7.5.3.10.1	Test Purpose and Environment	3165

A.7.5.3.10.2	Test Requirements	3169

A.7.5.3.11	PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR2 PCell	3170

A.7.5.3.11.1	PUCCH SCell activation with non-PUCCH SCell in a secondary PUCCH Group	3170

A.7.5.3.11.1.1	Test Purpose and Environment	3170

A.7.5.3.11.1.2	Test Requirements	3174

A.7.5.3.11.2	PUCCH SCell activation with non-PUCCH SCell in a primary PUCCH Group	3175

A.7.5.3.11.2.1	Test Purpose and Environment	3175

A.7.5.3.11.2.2	Test Requirements	3179

A.7.5.3.12	PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2	3180

A.7.5.3.12.1	Test Purpose and Environment	3180

A.7.5.3.12.2	Test Requirements	3184

A.7.5.3.13	SCell Activation for SCell in FR2 intra-band in non-DRX	3185

A.7.5.3.13.1	Test Purpose and Environment	3185

A.7.5.3.13.2	Test Requirements	3187

A.7.5.3.14	SCell Activation for known SCell in FR2 inter-band	3187

A.7.5.3.14.1	Test Purpose and Environment	3187

A.7.5.3.14.2	Test Requirements	3190

A.7.5.15	PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2	3191

A.7.5.15.3.1	Test Purpose and Environment	3191

A.7.5.3.2.2	Test Requirements	3193

A.7.5.4	Void	3194

A.7.5.5	Beam Failure Detection and Link recovery procedures	3194

A.7.5.5.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3194

A.7.5.5.1.1	Test Purpose and Environment	3194

A.7.5.5.1.2	Test Requirements	3198

A.7.5.5.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	3198

A.7.5.5.2.1	Test Purpose and Environment	3198

A.7.5.5.2.2	Test Requirements	3202

A.7.5.5.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	3202

A.7.5.5.3.1	Test Purpose and Environment	3202

A.7.5.5.3.2	Test Requirements	3206

A.7.5.5.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	3206

A.7.5.5.4.1	Test Purpose and Environment	3206

A.7.5.5.4.2	Test Requirements	3210

A.7.5.5.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3210

A.7.5.5.5.1	Test Purpose and Environment	3210

A.7.5.5.5.2	Test Requirements	3214

A.7.5.5.6	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	3214

A.7.5.5.6.1	Test Purpose and Environment	3214

A.7.5.5.6.2	Test Requirements	3218

A.7.5.5.7	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	3218

A.7.5.5.7.1	Test Purpose and Environment	3218

A.7.5.5.7.2	Test Requirements	3222

A.7.5.5.8	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode for UE fulfilling relaxed measurement criterion	3222

A.7.5.5.8.1	Test Purpose and Environment	3222

A.7.5.5.8.2	Test Requirements	3227

A.7.5.5.9	TRP specific Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	3227

A.7.5.5.9.1	Test Purpose and Environment	3227

A.7.5.5.9.2	Test Requirements	3233

A.7.5.5.10	TRP specific Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3233

A.7.5.5.10.1	Test Purpose and Environment	3233

A.7.5.5.10.2	Test Requirements	3239

A.7.5.5.11	Beam Failure Detection and Link Recovery Test for FR2-2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	3239

A.7.5.5.11.1	Test Purpose and Environment	3239

A.7.5.5.11.2	Test Requirements	3244

A.7.5.5.12	Beam Failure Detection and Link Recovery Test for FR2-2 PCell configured with CSI-RS-based BFD and LR in DRX mode	3244

A.7.5.5.12.1	Test Purpose and Environment	3244

A.7.5.5.12.2	Test Requirements	3249

A.7.5.5.13	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2-2 PCell configured with SSB-based BFD and LR in non-DRX mode	3249

A.7.5.5.13.1	Test Purpose and Environment	3249

A.7.5.5.13.2	Test Requirements	3253

A.7.5.6	Active BWP switch	3253

A.7.5.6.1	DCI-based and Timer-based Active BWP Switch	3253

A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of SCell with non-DRX in SA	3253

A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of SCell with non-DRX in SA	3258

A.7.5.6.1.3	NR FR2 DL active BWP switch with non-DRX in SA	3263

A.7.5.6.1.3.1	Test Purpose and Environment	3263

A.7.5.6.1.3.2	Test Requirements	3266

A.7.5.6.1.4	NR FR2-2- NR FR2-2 DL active BWP switch of SCell with non-DRX in SA	3266

A.7.5.6.1.4.1	Test Purpose and Environment	3266

A.7.5.6.1.4.2	Test Requirements	3271

A.7.5.6.2	RRC-based Active BWP Switch	3272

A.7.5.6.2.1.1	Test Purpose and Environment	3272

A.7.5.6.2.1.2	Test Requirements	3275

A.7.5.6.2.2	NR FR2-2 DL active BWP switch of PCell with non-DRX in SA	3276

A.7.5.6.2.2.1	Test Purpose and Environment	3276

A.7.5.6.2.2.2	Test Requirements	3280

A.7.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	3281

A.7.5.6.3.1.1	Test Purpose and Environment	3281

A.7.5.6.3.1.2	Test Requirements	3284

A.7.5.6.4	SCell dormancy switch	3284

A.7.5.6.4.1	NR FR2 PCell SCell dormancy switch of single FR2 SCell inside active time	3284

A.7.5.6.4.1.1	Test Purpose and Environment	3284

A.7.5.6.4.1.2	Test Requirements	3289

A.7.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time	3289

A.7.5.6.4.2.1	 Test Purpose and Environment	3289

A.7.5.6.4.2.2	 Test Requirements	3294

A.7.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	3294

A.7.5.6.5.1	Active BWP switch on multiple SCells with non-DRX in SA	3294

A.7.5.6.5.2	NR FR2-2 Active BWP switch on multiple SCells with non-DRX in SA	3297

A.7.5.6.5.2.1	Test Purpose and Environment	3297

A.7.5.6.5.2.2	Test Requirements	3301

A.7.5.7	PSCell addition and release delay	3302

A.7.5.7.1	Addition and Release Delay of known NR PSCell in FR2-2	3302

A.7.5.7.1.1	Test Purpose and Environment	3302

A.7.5.7.1.2	Test Requirements	3305

A.7.5.7.2	Addition and Release Delay of unknown NR PSCell in FR2-2	3305

A.7.5.7.2.1	Test Purpose and Environment	3305

A.7.5.7.2.2	Test Requirements	3308

A.7.5.8	Active TCI state switch delay	3308

A.7.5.8.1	MAC-CE based active TCI state switch	3308

A.7.5.8.2	RRC based active TCI state switch	3312

A.7.5.8.3	MAC-CE based active TCI state switch for HST FR2 scenario	3316

A.7.5.8.3.1	NR PCell FR2 HST active TCI state switch for a known TCI state	3316

A.7.5.8.3.1.1	Test Purpose and Environment	3316

A.7.5.8.3.1.2	Test Requirements	3320

A.7.5.9	Uplink spatial relation switch delay	3321

A.7.5.9.1.1.1	Test Purpose and Environment	3321

A.7.5.9.1.1.2	Test Requirements	3324

A.7.5.9.2	RRC based spatial relation switch	3324

A.7.5.9.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	3324

A.7.5.9.2.1.1	Test Purpose and Environment	3324

A.7.5.9.2.1.2	Test Requirements	3327

A.7.5.10	UE specific CBW change	3327

A.7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	3327

A.7.5.10.1.1	Test Purpose and Environment	3327

A.7.5.10.1.2	Test Requirements	3330

A.7.5.11	UE UL carrier RRC reconfiguration Delay	3330

A.7.5.11.1	UE UL carrier RRC reconfiguration Delay	3330

A.7.5.11.1.1	Test Purpose and Environment	3330

A.7.5.11.1.2	Test Requirements	3334

A.7.5.12	Conditional PSCell addition and release delay (FR2 SA)	3334

A.7.5.12.1	Addition and Release Delay of PSCell	3334

A.7.5.12.1.1	Test purpose and environment	3334

A.7.5.12.1.2	Test Parameters	3334

A.7.5.12.1.3	Test Requirements	3337

A.7.5.13	 Unified TCI state switching delay	3337

A.7.5.13.1	MAC-CE based active joint TCI state switching	3337

A.7.5.13.1.1	NR PCell FR2 active joint TCI state switch for a known TCI state	3337

A.7.5.13.1.1.1	Test Purpose and Environment	3337

A.7.5.13.1.1.2	Test parameters	3338

A.7.5.13.1.1.3	Test Requirements	3340

A.7.5.13.2	 MAC-CE based active uplink TCI state switch	3340

A.7.5.13.2.1	 NR FR2 PCell uplink TCI state switch for a known TCI state	3340

A.7.5.13.2.1.1	Test Purpose and Environment	3340

A.7.5.13.2.1.2	Test parameters	3341

A.7.5.13.2.1.3	Test Requirements	3343

A.7.5.13.3	MAC-CE based active downlink TCI state switch	3343

A.7.5.13.3.1	NR PCell FR2 active downlink TCI state switch to cell with additional PCI for a known TCI state	3343

A.7.5.13.3.1.1	Test Purpose and Environment	3343

A.7.5.13.3.1.2	Test Parameters	3344

A.7.5.13.3.1.3	Test Requirements	3347

A.7.6	Measurement procedure	3347

A.7.6.1	Intra-frequency Measurements	3347

A.7.6.1.1	SA event triggered reporting test without gap under non-DRX	3347

A.7.6.1.1.1	Test purpose and Environment	3347

A.7.6.1.1.2	Test Requirements	3351

A.7.6.1.2	SA event triggered reporting test without gap under DRX	3351

A.7.6.1.2.1	Test purpose and Environment	3351

A.7.6.1.2.2	Test Requirements	3354

A.7.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	3355

A.7.6.1.3.1	Test purpose and Environment	3355

A.7.6.1.3.2	Test Requirements	3358

A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	3358

A.7.6.1.4.1	Test purpose and Environment	3358

A.7.6.1.4.2	Test Requirements	3361

A.7.6.1.5	SA event triggered reporting test without gap under non-DRX for UE configured with highSpeedMeasCA-Scell-r17	3362

A.7.6.1.5.1	Test purpose and Environment	3362

A.7.6.1.5.2	Test Requirements	3365

A.7.6.1.6	SA event triggered reporting test without gap under non-DRX for FR2-2	3365

A.7.6.1.6.1	Test purpose and Environment	3365

A.7.6.1.6.2	Test Requirements	3368

A.7.6.1.7	SA event triggered reporting test without gap under DRX for FR2-2	3369

A.7.6.1.7.1	Test purpose and Environment	3369

A.7.6.1.7.2	Test Requirements	3371

A.7.6.1.8	SA event triggered reporting test with per-UE gaps under non-DRX for FR2-2	3372

A.7.6.1.8.1	Test purpose and Environment	3372

A.7.6.1.8.2	Test Requirements	3375

A.7.6.1.9	SA event triggered reporting test with per-UE gaps under DRX for FR2-2	3376

A.7.6.1.9.1	Test purpose and Environment	3376

A.7.6.1.9.2	Test Requirements	3378

A.7.6.1.10	SA event triggered reporting test with SSB time index detection without gap under non-DRX for FR2-2	3379

A.7.6.1.10.1	Test purpose and Environment	3379

A.7.6.1.10.2	Test Requirements	3382

A.7.6.1.11	SA event triggered reporting test with SSB time index detection with per-UE gaps under non-DRX for FR2-2	3383

A.7.6.1.11.1	Test purpose and Environment	3383

A.7.6.1.11.2	Test Requirements	3385

A.7.6.2	Inter-frequency Measurements	3386

A.7.6.2.1	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR2)	3386

A.7.6.2.1.1	Test Purpose and Environment	3386

A.7.6.2.1.2	Test Requirements	3389

A.7.6.2.2	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (Pcell in FR2)	3389

A.7.6.2.2.1	Test Purpose and Environment	3389

A.7.6.2.2.2	Test Requirements	3393

A.7.6.2.3	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR2)	3393

A.7.6.2.3.1	Test Purpose and Environment	3393

A.7.6.2.3.2	Test Requirements	3397

A.7.6.2.4	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (Pcell in FR2)	3397

A.7.6.2.4.1	Test Purpose and Environment	3397

A.7.6.2.4.2	Test Requirements	3401

A.7.6.2.5	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)	3401

A.7.6.2.5.1	Test Purpose and Environment	3401

A.7.6.2.5.2	Test Requirements	3405

A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (Pcell in FR1)	3406

A.7.6.2.6.1	Test Purpose and Environment	3406

A.7.6.2.6.2	Test Requirements	3410

A.7.6.2.7	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)	3411

A.7.6.2.7.1	Test Purpose and Environment	3411

A.7.6.2.7.2	Test Requirements	3415

A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)	3416

A.7.6.2.8.1	Test Purpose and Environment	3416

A.7.6.2.8.2	Test Requirements	3420

A.7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	3421

A.7.6.2.9.1	Test Purpose and Environment	3421

A.7.6.2.9.2	Test Requirements	3425

A.7.6.2.10	SA event triggered reporting test without gap under non-DRX	3425

A.7.6.2.10.1	Test Purpose and Environment	3425

A.7.6.2.10.2	Test Requirements	3427

A.7.6.2.11	SA event triggered reporting test without gap under DRX	3427

A.7.6.2.11.1	Test Purpose and Environment	3427

A.7.6.2.11.2	Test Requirements	3430

A.7.6.2.12	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is not used (PCell in FR2-2)	3430

A.7.6.2.12.1	Test Purpose and Environment	3430

A.7.6.2.12.2	Test Requirements	3434

A.7.6.2.13	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is used (PCell in FR2-2)	3435

A.7.6.2.13.1	Test Purpose and Environment	3435

A.7.6.2.13.2	Test Requirements	3439

A.7.6.2.14	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is not used (PCell in FR2-2)	3440

A.7.6.2.14.1	Test Purpose and Environment	3440

A.7.6.2.14.2	Test Requirements	3444

A.7.6.2.15	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is used (PCell in FR2-2)	3445

A.7.6.2.15.1	Test Purpose and Environment	3445

A.7.6.2.15.2	Test Requirements	3449

A.7.6.2.16	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is not used (PCell in FR1)	3450

A.7.6.2.16.1	Test Purpose and Environment	3450

A.7.6.2.16.2	Test Requirements	3456

A.7.6.2.17	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is used (PCell in FR1)	3456

A.7.6.2.17.1	Test Purpose and Environment	3456

A.7.6.2.17.2	Test Requirements	3463

A.7.6.2.18	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is not used (PCell in FR1)	3464

A.7.6.2.18.1	Test Purpose and Environment	3464

A.7.6.2.18.2	Test Requirements	3470

A.7.6.2.19	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is used (PCell in FR1)	3470

A.7.6.2.19.1	Test Purpose and Environment	3470

A.7.6.2.19.2	Test Requirements	3478

A.7.6.3	L1-RSRP measurement for beam reporting	3479

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	3479

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	3479

A.7.6.3.1.1	Test Purpose and Environment	3479

A.7.6.3.1.2	Test parameters	3479

A.7.6.3.1.3	Test Requirements	3481

A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used	3481

A.7.6.3.2.1	Test Purpose and Environment	3481

A.7.6.3.2.2	Test parameters	3482

A.7.6.3.2.3	Test Requirements	3484

A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	3484

A.7.6.3.3.1	Test Purpose and Environment	3484

A.7.6.3.3.2	Test parameters	3485

A.7.6.3.3.3	Test Requirements	3487

A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	3488

A.7.6.3.4.1	Test Purpose and Environment	3488

A.7.6.3.4.2	Test parameters	3488

A.7.6.3.3.3	Test Requirements	3490

A.7.6.3.5	SSB based L1-RSRP measurement when DRX is used for power class 6 UE configured with highSpeedMeasFlagFR2-r17	3491

A.7.6.3.5.1	Test Purpose and Environment	3491

A.7.6.3.5.2	Test parameters	3491

A.7.6.3.5.3	Test Requirements	3493

A.7.6.3.6	Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used	3493

A.7.6.3.6.1	Test Purpose and Environment	3493

A.7.6.3.6.2	Test parameters	3494

A.7.6.3.6.3	Test Requirements	3497

A.7.6.4	CLI measurements	3497

A.7.6.4.1	SRS-RSRP measurement with non-DRX	3497

A.7.6.4.1.1	Test Purpose and Environment	3497

A.7.6.4.1.2	Test Parameters	3498

A.7.6.4.1.3	Test Requirements	3500

A.7.6.4.2	CLI-RSSI measurement with non-DRX	3500

A.7.6.4.2.1	Test Purpose and Environment	3500

A.7.6.4.2.2	Test Parameters	3501

A.7.6.4.2.3	Test Requirements	3502

A.7.6.5.1   SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2)	3503

A.7.6.5.1.1	Test Purpose and Environment	3503

A.7.6.5.1.2	Test Requirements	3506

A.7.6.6	L1-SINR measurement for beam reporting	3506

A.7.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	3508

A.7.6.6.2.1	Test Purpose and Environment	3508

A.7.6.6.2.2	Test parameters	3509

A.7.6.6.2.3	Test Requirements	3511

A.7.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	3511

A.7.6.6.3.1	Test Purpose and Environment	3511

A.7.6.6.3.2	Test parameters	3512

A.7.6.6.3.3	Test Requirements	3514

A.7.6.7	CSI-RS based intra-frequency Measurements	3514

A.7.6.7.1	SA event triggered reporting test without gap under DRX for CSI-RS based intra-frequency measurement	3514

A.7.6.7.1.1	Test purpose and Environment	3514

A.7.6.7.1.2	Test Requirements	3517

A.7.6.8	CSI-RS based inter-frequency Measurements	3517

A.7.6.8.1	SA event triggered reporting tests for FR2 CSI-RS based measurement when non-DRX is used (PCell in FR2)	3517

A.7.6.8.1.1	Test Purpose and Environment	3517

A.7.6.8.1.2	Test Requirements	3520

A.7.6.9	RSTD measurements	3520

A.7.6.9.1	 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA	3520

A.7.6.9.1.1	Test Purpose and Environment	3520

A.7.6.9.1.2	Test Requirements	3528

A.7.6.9.2	 NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR2 SA	3528

A.7.6.9.2.1	Test Purpose and Environment	3528

A.7.6.9.2.2	Test Requirements	3535

A.7.6.9.3	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA	3535

A.7.6.9.3.1	Test Purpose and Environment	3535

A.7.6.9.3.2	Test Requirements	3540

A.7.6.9.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA without measurement gap	3540

A.7.6.9.4.1	Test Purpose and Environment	3540

A.7.6.9.4.2	Test Requirements	3544

A.7.6.9.5	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_CONNECTED state with Rx TEG	3544

A.7.6.9.5.1	Test Purpose and Environment	3544

A.7.6.9.5.2	Test Requirements	3548

A.7.6.10 PRS-RSRP measurements	3548

A.7.6.10.1 PRS-RSRP reporting delay test case for single positioning frequency layer	3548

A.7.6.10.1.1	Test Purpose and Environment	3548

A.7.6.10.1.2	Test Requirements	3552

A.7.6.10.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	3552

A.7.6.10.2.1	Test Purpose and Environment	3552

A.7.6.10.2.2	Test Requirements	3556

A.7.6.10.3	PRS-RSRP reporting delay test case for reduced number of samples	3556

A.7.6.10.3.1	Test Purpose and Environment	3556

A.7.6.10.3.2	Test Requirements	3560

A.7.6.10.4	PRS-RSRP reporting delay test case for single positioning frequency layer outside MG	3560

A.7.6.10.4.1	Test Purpose and Environment	3560

A.7.6.10.4.2	Test Requirements	3564

A.7.6.11	UE Rx-Tx time difference measurements	3564

A.7.6.11.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	3564

A.7.6.11.1.1	Test purpose and environment	3564

A.7.6.11.1.2	Test requirements	3568

A.7.6.11.2	UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA	3568

A.7.6.11.2.1	Test purpose and environment	3568

A.7.6.11.2.2	Test requirements	3572

A.7.6.11.3	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA with reduced sample number	3572

A.7.6.11.3.1	Test purpose and environment	3572

A.7.6.11.3.2	Test requirements	3574

A.7.6.11.4	UE Rx-Tx time difference measurements without gaps in FR2 SA	3575

A.7.6.11.4.1	Test purpose and environment	3575

A.7.6.11.4.2	Test requirements	3577

A.7.6.11.5	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA with multiple RxTx TEGs	3578

A.7.6.11.5.1	Test purpose and environment	3578

A.7.6.11.5.2	Test requirements	3580

A.7.6.12	PRS-RSRPP measurements	3581

A.7.6.12.1 PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_CONNECTED state	3581

A.7.6.12.1.1	Test Purpose and Environment	3581

A.7.6.12.1.2	Test Requirements	3583

A.7.6.12.2	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_CONNECTED state	3584

A.7.6.12.2.1	Test Purpose and Environment	3584

A.7.6.12.2.2	Test Requirements	3586

A.7.6.12.3	PRS-RSRPP reporting delay test case for gapless measurement in FR2	3586

A.7.6.12.3.1	Test Purpose and Environment	3586

A.7.6.12.3.2	Test Requirements	3590

A.7.6.13	UE Rx-Tx time difference measurements for PDC	3590

A.7.6.13.1	UE Rx-Tx time difference measurement for propagation delay compensation using PRS in FR2	3590

A.7.6.13.1.1	Test purpose and environment13	3590

A.7.6.13.1.2	Test requirements	3592

A.7.6.13.2	UE Rx-Tx time difference measurement for propagation delay compensation using TRS in FR2	3593

A.7.6.13.2.1	Test purpose and environment	3593

A.7.6.13.2.2	Test requirements	3594

A.7.6.14	SA event triggered reporting tests with Pre-MG	3595

A.7.6.14.1	Intra-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MG in FR2	3595

A.7.6.14.1.1	Test purpose and Environment	3595

A.7.6.14.1.2	Test parameters	3595

A.7.6.14.1.3	Test Requirements	3597

A.7.6.14.2	Intra-frequency measurement test with SA event triggered reporting tests: with network-controlled activation/deactivation of Pre-MG in FR2	3598

A.7.6.14.2.1	Test purpose and Environment	3598

A.7.6.14.2.2	Test parameters	3598

A.7.6.14.2.3	Test Requirements	3600

A.7.6.15	SA event triggered reporting tests with concurrent gaps	3601

A.7.6.15.1	SA event triggered reporting tests For FR2 with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements	3601

A.7.6.15.1.1	Test Purpose and Environment	3601

A.7.6.15.1.2	Test Requirements	3604

A.7.6.15.2	SA event triggered reporting tests For FR2 with concurrent measurement gaps without SSB time index detection when DRX is not used (PCell in FR2)	3604

A.7.6.15.2.1	Test Purpose and Environment	3604

A.7.6.15.2.2	Test Requirements	3607

A.7.6.15.3	SA event triggered reporting tests for FR2 concurrent gap with partially partial overalpping scenario for SSB-based measurements and PRS-based measurement	3607

A.7.6.15.3.1	Test Purpose and Environment	3607

A.7.6.15.3.2	Test Requirements	3611

A.7.6.16	SA event triggered reporting tests with NCSG	3611

A.7.6.16.1	SA event triggered reporting test with per-UE NCSG under non-DRX	3611

A.7.6.16.1.1	Test purpose and Environment	3611

A.7.6.16.1.2	Test Requirements	3615

A.7.6.16.2	SA event triggered reporting tests on inter-frequency measurement with NCSG for FR2 when DRX is not used (PCell in FR2)	3615

A.7.6.16.2.1	Test Purpose and Environment	3615

A.7.6.16.2.2	Test Requirements	3618

A.7.6.16.3	Event triggered reporting test on deactivated Scell measurement via NCSG in FR2 in non-DRX	3618

A.7.6.16.3.1	Test Purpose and Environment	3618

A.7.6.16.3.2	Test Requirements	3621

A.7.7	Measurement Performance requirements	3621

A.7.7.1	SS-RSRP	3622

A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3622

A.7.7.1.1.1	Test Purpose and Environment	3622

A.7.7.1.1.2	Test parameters	3622

A.7.7.1.1.3	Test Requirements	3626

A.7.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3626

A.7.7.1.2.1	Test Purpose and Environment	3626

A.7.7.1.2.2	Test parameters	3627

A.7.7.1.2.3	Test Requirements	3631

A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	3632

A.7.7.1.3.1	Test Purpose and Environment	3632

A.7.7.1.3.2	Test parameters	3632

A.7.7.1.3.3	Test Requirements	3636

A.7.7.2	SS-RSRQ	3636

A.7.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	3636

A.7.7.2.1.1	Test Purpose and Environment	3636

A.7.7.2.1.2	Test Parameters	3636

A.7.7.2.1.3	Test Requirements	3638

A.7.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3638

A.7.7.2.2.1	Test Purpose and Environment	3638

A.7.7.2.2.2	Test Parameters	3638

A.7.7.2.2.3	Test Requirements	3640

A.7.7.3	SS-SINR	3640

A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3640

A.7.7.3.1.1	Test Purpose and Environment	3640

A.7.7.3.1.2	Test Parameters	3640

A.7.7.3.1.3	Test Requirements	3642

A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3642

A.7.7.3.2.1	Test Purpose and Environment	3642

A.7.7.3.2.2	Test Parameters	3642

A.7.7.3.2.3	Test Requirements	3644

A.7.7.4	L1-RSRP measurement for beam reporting	3644

A.7.7.4.1	SSB based L1-RSRP measurement	3644

A.7.7.4.1.1	Test Purpose and Environment	3644

A.7.7.4.1.2	Test parameters	3645

A.7.7.4.1.3	Test Requirements	3647

A.7.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	3648

A.7.7.4.2.1	Test Purpose and Environment	3648

A.7.7.4.2.2	Test parameters	3648

A.7.7.4.2.3	Test Requirements	3650

A.7.7.5	CLI measurements	3651

A.7.7.5.1	SA SRS-RSRP measurement accuracy with FR2 serving cell	3651

A.7.7.5.1.1	Test Purpose and Environment	3651

A.7.7.5.1.2	Test parameters	3651

A.7.7.5.1.3	Test Requirements	3654

A.7.7.5.2	SA CLI-RSSI measurement accuracy with FR2 serving cell	3655

A.7.7.5.2.1	Test Purpose and Environment	3655

A.7.7.5.2.2	Test parameters	3655

A.7.7.5.2.3	Test Requirements	3657

A.7.7.6	L1-SINR measurement for beam reporting	3658

A.7.7.6.1.1	Test Purpose and Environment	3658

A.7.7.6.1.2	Test parameters	3658

A.7.7.6.1.3	Test Requirements	3660

A.7.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	3661

A.7.7.6.2.1	Test Purpose and Environment	3661

A.7.7.6.2.2	Test parameters	3661

A.7.7.6.2.3	Test Requirements	3663

A.7.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	3663

A.7.7.6.3.1	Test Purpose and Environment	3663

A.7.7.6.3.2	Test parameters	3664

A.7.7.6.3.3	Test Requirements	3666

A.7.7.7	CSI-RSRP	3666

A.7.7.7.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3666

A.7.7.7.1.1	Test Purpose and Environment	3666

A.7.7.7.1.2	Test parameters	3667

A.7.7.7.1.3	Test Requirements	3670

A.7.7.7.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3671

A.7.7.7.2.1	Test Purpose and Environment	3671

A.7.7.7.2.2	Test parameters	3671

A.7.7.7.2.3	Test Requirements	3674

A.7.7.8	CSI-RSRQ	3674

A.7.7.8.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	3674

A.7.7.8.1.1	Test Purpose and Environment	3674

A.7.7.8.1.2	Test Parameters	3675

A.7.7.8.1.3	Test Requirements	3677

A.7.7.8.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3677

A.7.7.8.2.1	Test Purpose and Environment	3677

A.7.7.8.2.2	Test Parameters	3677

A.7.7.8.2.3	Test Requirements	3679

A.7.7.9	CSI-SINR	3679

A.7.7.9.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3679

A.7.7.9.1.1	Test Purpose and Environment	3679

A.7.7.9.1.2	Test Parameters	3679

A.7.7.9.1.3	Test Requirements	3681

A.7.7.9.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3681

A.7.7.9.2.1	Test Purpose and Environment	3681

A.7.7.9.2.2	Test Parameters	3682

A.7.7.9.2.3	Test Requirements	3684

A.7.7.10	RSTD measurements	3684

A.7.7.10.1	RSTD measurement accuracy test case for single positioning frequency layer	3684

A.7.7.10.1.1	Test purpose and Environment	3684

A.7.7.10.1.2	Test Requirements	3687

A.7.7.10.2	RSTD measurement accuracy test case for dual positioning frequency layer	3687

A.7.7.10.2.1	Test purpose and Environment	3687

A.7.7.10.2.2	Test Requirements	3691

A.7.7.10.3	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR2 in RRC_CONNECTED state	3692

A.7.7.10.3.1	Test purpose and Environment	3692

A.7.7.10.3.2	Test Requirements	3693

A.7.7.10.4	RSTD measurement accuracy test case with Rx TEG	3694

A.7.7.10.4.1	Test purpose and Environment	3694

A.7.7.10.4.2	Test Requirements	3696

A.7.7.11	PRS-RSRP measurements	3696

A.7.7.11.1	SA measurement accuracy with PRS in FR2	3696

A.7.7.11.1.1	Test Purpose and Environment	3696

A.7.7.11.1.2	Test parameters	3696

A.7.7.11.1.3	Test Requirements	3700

A.7.7.11.2	SA measurement accuracy with PRS in FR2 with reduced sample number	3701

A.7.7.11.2.1	Test Purpose and Environment	3701

A.7.7.11.2.2	Test parameters	3701

A.7.7.11.2.3	Test Requirements	3703

A.7.7.12	UE Rx-Tx time difference measurements	3704

A.7.7.12.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA	3704

A.7.7.12.1.1	Test purpose and environment	3704

A.7.7.12.1.2	Test parameters	3704

A.7.7.12.1.3	Test requirements	3707

A.7.7.12.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA	3707

A.7.7.12.2.1	Test purpose and environment	3707

A.7.7.12.2.2	Test parameters	3708

A.7.7.12.2.3	Test requirements	3711

A.7.7.12.3	UE Rx-Tx time difference measurement accuracy with RxTx TEG	3711

A.7.7.12.3.1	Test purpose and environment	3711

A.7.7.12.3.2	Test parameters	3711

A.7.7.12.3.3	Test requirements	3714

A.7.7.13	PRS-RSRPP measurements	3714

A.7.7.13.1	SA measurement accuracy with PRS in FR2	3714

A.7.7.13.1.1	Test Purpose and Environment	3714

A.7.7.13.1.2	Test parameters	3714

A.7.7.13.1.3	Test Requirements	3717

A.7.7.13.2	SA measurement accuracy with reduced PRS samples in FR2	3718

A.7.7.13.2.1	Test Purpose and Environment	3718

A.7.7.13.2.2	Test parameters	3718

A.7.7.13.2.3	Test Requirements	3720

A.7.8	Measurement procedure in RRC_INACTIVE	3721

A.7.8.1	RSTD measurements	3721

A.7.8.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_INACTIVE state	3721

A.7.8.1.1.1	Test Purpose and Environment	3721

A.7.8.1.1.2	Test Requirements	3724

A.7.8.1.2	NR RSTD measurement reporting delay test case with reduced number of samples in RRC_INACTIVE, FR1 SA	3724

A.7.8.1.2.1	Test Purpose and Environment	3724

A.7.8.1.2.2	Test Requirements	3730

A.7.8.2	PRS-RSRP measurements	3731

A.7.8.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	3731

A.7.8.2.1.1	Test Purpose and Environment	3731

A.7.8.2.1.2	Test Requirements	3735

A.7.8.2.2	PRS-RSRP reporting delay test case with reduced number of samples in RRC_INACTIVE	3735

A.7.8.2.2.1	Test purpose and Environment	3735

A.7.8.2.2.2	Test Requirements	3739

A.7.8.3	UE Rx-Tx time difference measurements	3739

A.7.8.3.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	3739

A.7.8.3.1.1	Test purpose and environment	3739

A.7.8.3.1.2	Test requirements	3743

A.7.8.3.2	UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE, FR2 SA	3743

A.7.8.3.2.1	Test purpose and environment	3743

A.7.8.3.2.2	Test requirements	3745

A.7.8.4	PRS-RSRPP measurements	3746

A.7.8.4.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_INACTIVE state	3746

A.7.8.4.1.1	Test Purpose and Environment	3746

A.7.8.4.1.2	Test Requirements	3748

A.7.8.4.2	PRS-RSRPP reporting delay test with reduced number of samples for single positioning frequency layer in FR2 in RRC_INACTIVE state	3748

A.7.8.4.2.1	Test Purpose and Environment	3748

A.7.8.4.2.2	Test Requirements	3751

A.7.9	Measurement performance requirements in RRC_INACTIVE	3751

A.7.9.1	RSTD measurements	3751

A.7.9.1.1	RSTD measurement accuracy test case for single positioning frequency layer in FR2 in RRC_INACTIVE state	3751

A.7.9.1.1.1	Test purpose and Environment	3751

A.7.9.1.1.2	Test Requirements	3753

A.7.9.1.2	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR2 in RRC_INACTIVE state	3753

A.7.9.1.2.1	Test purpose and Environment	3753

A.7.9.1.2.2	Test Requirements	3755

A.7.9.2	PRS-RSRP measurements	3755

A.7.9.2.1	SA measurement accuracy with PRS in FR2 in RRC_INACTIVE	3755

A.7.9.2.1.1	Test Purpose and Environment	3755

A.7.9.2.1.2	Test parameters	3755

A.7.9.2.1.3	Test Requirements	3757

A.7.9.2.2	PRS-RSRP measurements with reduced number of sample in RRC_INACTIVE	3758

A.7.9.2.2.1	Test Purpose and Environment	3758

A.7.9.2.2.2	Test parameters	3758

A.7.9.2.2.3	Test Requirements	3760

A.7.9.3	UE Rx-Tx time difference measurements	3761

A.7.9.3.1	UE Rx-Tx time difference measurements in RRC_INACTIVE	3761

A.7.9.3.1.1	Test purpose and environment	3761

A.7.9.3.1.2	Test parameters	3761

A.7.9.3.1.3	Test requirements	3763

A.7.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA	3763

A.7.9.3.2.1	Test purpose and environment	3763

A.7.9.3.2.2	Test parameters	3763

A.7.9.3.2.3	Test requirements	3766

A.7.9.4	PRS-RSRPP measurements	3766

A.7.9.4.1	SA measurement accuracy in FR2 in RRC INACTIVE	3766

A.7.9.4.1.1	Test Purpose and Environment	3766

A.7.9.4.1.2	Test parameters	3766

A.7.9.4.1.3	Test Requirements	3768

A.7.9.4.2	SA measurement accuracy with reduced PRS samples in FR2 in RRC INACTIVE	3769

A.7.9.4.2.1	Test Purpose and Environment	3769

A.7.9.4.2.2	Test parameters	3769

A.7.9.4.2.3	Test Requirements	3771

A.8	E-UTRA standalone tests for NR RRM	3717

A.8.1	Void	3717

A.8.2	RRC_IDLE state mobility	3717

A.8.2.1	Inter-RAT NR Cell re-selection	3717

A.8.2.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	3717

A.8.2.1.1.1	Test Purpose and Environment	3717

A.8.2.1.1.2	Test Requirements	3722

A.8.2.1.2	E-UTRA Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	3723

A.8.2.1.2.1	Test Purpose and Environment	3723

A.8.2.1.2.2	Test Requirements	3727

A.8.2.2	E-UTRA – NR Inter-RAT Early Measruement Reporting	3728

A.8.2.2.1	E-UTRA – NR Early Measurement Reporting for NR in FR1	3728

A.8.2.2.1.1	Test Purpose and Environment	3728

A.8.2.2.1.2	Test Requirements	3732

A.8.2.2.2	E-UTRA – NR Early Measurement Reporting for NR in FR2	3732

A.8.2.2.2.1	Test Purpose and Environment	3732

A.8.2.2.2.2	Test Requirements	3735

A.8.3	RRC_CONNECTED state mobility	3736

A.8.3.1	Handover	3736

A.8.3.1.1	E-UTRAN - NR handover in FR1	3736

A.8.3.1.1.1	Test Purpose and Environment	3736

A.8.3.1.1.2	Test Requirements	3741

A.8.4	Measurement procedure	3741

A.8.4.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay	3741

A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX	3741

A.8.4.1.1.1	Test Purpose and Environment	3741

A.8.4.1.1.2	Test Requirements	3743

A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX	3744

A.8.4.1.2.1	Test Purpose and Environment	3744

A.8.4.1.2.2	Test Requirements	3745

A.8.4.2	E-UTRA – NR Inter-RAT Measurements	3745

A.8.4.2.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	3745

A.8.4.2.1.1	Test Purpose and Environment	3745

A.8.4.2.1.2	Test Requirements	3750

A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	3750

A.8.4.2.2.1	Test Purpose and Environment	3750

A.8.4.2.2.2	Test Requirements	3754

A.8.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	3754

A.8.4.2.3.1	Test Purpose and Environment	3754

A.8.4.2.3.2	Test Requirements	3759

A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	3759

A.8.4.2.4.1	Test Purpose and Environment	3759

A.8.4.2.4.2	Test Requirements	3763

A.8.4.2.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	3763

A.8.4.2.5.1	Test Purpose and Environment	3763

A.8.4.2.5.2	Test Requirements	3765

A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	3766

A.8.4.2.6.1	Test Purpose and Environment	3766

A.8.4.2.6.2	Test Requirements	3768

A.8.4.2.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	3769

A.8.4.2.7.1	Test Purpose and Environment	3769

A.8.4.2.7.2	Test Requirements	3771

A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	3772

A.8.4.2.8.1	Test Purpose and Environment	3772

A.8.4.2.8.2	Test Requirements	3774

A.8.4.2.9	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	3775

A.8.4.2.9.1	Test Purpose and Environment	3775

A.8.4.2.9.2	Test Requirements	3779

A.8.5	Measurement performance	3779

A.8.5.1	SFTD accuracy	3779

A.8.5.1.1	SFTD accuracy	3779

A.8.5.1.1.1	Test Purpose	3779

A.8.5.1.1.2	Test Environment	3780

A.8.5.1.1.3	Test Requirements	3786

A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements	3786

A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	3786

A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	3791

A.8.5.2.1.2.1	Test Purpose and Environment	3791

A.8.5.2.1.2.2	Test Parameters	3791

A.8.5.2.1.2.3	Test Requirements	3793

A.8.5.2.2	SS-RSRQ	3793

A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	3793

A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	3798

A.8.5.2.2.2.1	Test Purpose and Environment	3798

A.8.5.2.2.2.2	Test Parameters	3798

A.8.5.2.2.2.3	Test Requirements	3800

A.8.5.2.3	SS-SINR	3800

A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	3800

A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	3804

A.8.5.2.3.2.1	Test Purpose and Environment	3804

A.8.5.2.3.2.2	Test Parameters	3804

A.8.5.2.3.2.3	Test Requirements	3806

A.9	V2X Tests	3807

A.9.1	V2X Tests in FR1	3807

A.9.1.1	Test for V2X UE Transmit Timing	3807

A.9.1.1.1 Test for GNSS as Synchronization Reference Source	3807

A.9.1.1.1.1	Test Purpose and Environment	3807

A.9.1.1.1.2	Test requirements	3807

A.9.1.1.2	Test for SyncRef UE as Synchronization Reference Source	3807

A.9.1.1.2.1	Test Purpose and Environment	3807

A.9.1.1.2.2	Test requirements	3808

A.9.1.1.3	Test for FR1 NR Cell as Synchronization Reference Source	3808

A.9.1.1.3.1	Test Purpose and Environment	3808

A.9.1.1.3.2	Test requirements	3811

A.9.1.2	Test for Initiation/Cease of S-SSB Transmission with V2X Sidelink Communication	3811

A.9.1.2.1	Test for FR1 NR Cell as synchronization reference source without gap under non-DRX	3811

A.9.1.2.1.1	Test Purpose and Environment	3811

A.9.1.2.1.2	Test Requirements	3814

A.9.1.2.2	Test for SyncRef UE as synchronization reference source	3814

A.9.1.2.2.1	Test Purpose and Environment	3814

A.9.1.2.2.2	Test Requirements	3816

A.9.1.2.3	Test for SyncRef UE as synchronization reference source when SL-DRX is used	3816

A.9.1.2.3.1	Test Purpose and Environment	3816

A.9.1.2.3.2	Test Requirements	3818

A.9.1.3	 Test for V2X Synchronization Reference Selection/Reselection	3818

A.9.1.3.1	 Test for GNSS configured as the highest priority	3818

A.9.1.3.1.1	Test Purpose and Environment	3818

A.9.1.3.1.2	Test Requirements	3820

A.9.1.3.2	 Test for FR1 NR Cell configured as the highest priority	3821

A.9.1.3.2.1	Test Purpose and Environment	3821

A.9.1.3.2.2	Test Requirements	3823

A.9.1.3.3	Test for GNSS configured as the highest priority under SL-DRX	3824

A.9.1.3.3.1	Test Purpose and Environment	3824

A.9.1.3.3.2	Test Requirements	3826

A.9.1.3.4	Test for FR1 NR Cell configured as the highest priority under SL-DRX	3827

A.9.1.3.4.1	Test Purpose and Environment	3827

A.9.1.3.4.2	Test Requirements	3829

A.9.1.4	Test for L1 SL-RSRP Measurement	3830

A.9.1.4.1	Test for V2X UE Autonomous Resource Selection/Reselection	3830

A.9.1.4.1.1	Test Purpose and Environment	3830

A.9.1.4.1.2	Test Requirements	3833

A.9.1.4.2	Test for V2X UE Resource Pre-emption	3833

A.9.1.4.2.1	Test Purpose and Environment	3833

A.9.1.4.2.2	Test Requirements	3837

A.9.1.4.3	 Test for V2X UE Resource Re-evaluation	3837

A.9.1.4.3.1	Test Purpose and Environment	3837

A.9.1.4.3.2	Test Requirements	3844

A.9.1.4.4	Test for V2X UE Autonomous Resource Selection/Reselection with Periodic Sensing	3844

A.9.1.4.4.1	Test Purpose and Environment	3844

A.9.1.4.4.2	Test Requirements	3847

A.9.1.4.5	Test for V2X UE Autonomous Resource Selection/Reselection with Contiguous Sensing	3847

A.9.1.4.5.1	Test Purpose and Environment	3847

A.9.1.4.5.2	Test Requirements	3850

A.9.1.4.6	Test for V2X UE Autonomous Resource Selection/Reselection in SL-DRX	3850

A.9.1.4.6.1	Test Purpose and Environment	3850

A.9.1.4.6.2	Test Requirements	3853

A.9.1.5	Test for Congestion Control Measurement	3853

A.9.1.5.1	Test Purpose and Environment	3853

A.9.1.5.2	Test Requirements	3859

A.9.1.6	Test for Interruption	3859

A.9.1.6.1	Test for Interruption to WAN due to V2X Sidelink Communication	3859

A.9.1.6.1.1	Test Purpose and Environment	3859

A.9.1.6.1.2	Test Requirements	3862

A.9.1.6.2	Test for interruption to WAN at transitions between active and non-active during SL-DRX in asynchronous case	3862

A.9.1.6.2.1	Test Purpose and Environment	3862

A.9.1.6.2.2	Test Requirements	3866

A.9.1.6.3	Test for Interruption at NR Sidelink Diccovery Configuration	3866

A.9.1.6.3.1	Test Purpose and Environment	3866

A.9.1.6.3.2	Test Requirements	3869

A.9.1.7	Selection / Reselection of relay UE	3869

A.9.1.7.1	Test Purpose and Environment	3869

A.9.1.7.2	Test Requirements	3875

A.10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	3877

A.10.1	RRC_CONNECTED state mobility	3877

A.10.1.1	RRC connection mobility control	3877

A.10.1.1.1	Random Access	3877

A.10.1.1.1.1	4-step RA type contention-based random access for NR PSCell with CCA	3877

A.10.1.1.1.1.1	Test Purpose and Environment	3877

A.10.1.1.1.1.2	Test Requirements	3880

A.10.1.1.1.1.2.1	Random Access Preamble Transmission	3880

A.10.1.1.1.1.2.2	Random Access Response Reception	3880

A.10.1.1.1.1.2.3	No Random Access Response Reception	3881

A.10.1.1.1.1.2.4	Receiving an UL grant for msg3 retransmission	3881

A.10.1.1.1.1.2.5	 Contention Resolution Timer expiry	3881

A.10.1.1.1.2	4-step RA type non-contention based random access for NR PSCell with CCA	3882

A.10.1.1.1.2.1	Test Purpose and Environment	3882

A.10.1.1.1.2.2	Test Requirements	3885

A.10.1.1.1.2.2.1	SSB-based Random Access Preamble Transmission	3885

A.10.1.1.1.2.2.2	Random Access Response Reception	3886

A.10.1.1.1.2.2.3	No Random Access Response Reception	3886

A.10.1.1.1.3	2-step RA type contention-based random access for NR PSCell with CCA	3886

A.10.1.1.1.3.1	Test Purpose and Environment	3886

A.10.1.1.1.3.2	Test Requirements	3890

A.10.1.1.1.3.2.1	MsgA Transmission	3890

A.10.1.1.1.3.2.2	MsgB Reception	3890

A.10.1.1.1.3.2.3	No MsgB Reception	3891

A.10.1.1.1.4	2-step RA type non-contention based random access for NR PSCell with CCA	3891

A.10.1.1.1.4.1	Test Purpose and Environment	3891

A.10.1.1.1.4.2	Test Requirements	3895

A.10.1.1.1.4.2.1	MsgA Transmission	3895

A.10.1.1.1.4.2.2	MsgB Reception	3896

A.10.1.1.1.4.2.3	No MsgB Reception	3896

A.10.1.2	Handover with PSCell from EN-DC to EN-DC with known target PSCell using CCA	3896

A.10.1.2.1	Test Purpose and Environment	3896

A.10.1.2.2	Test Requirements	3902

A.10.2	Timing	3903

A.10.2.1	UE transmit timing	3903

A.10.2.1.1	UE Transmit Timing Test with PSCell under DL CCA	3903

A.10.2.1.1.1	Test Purpose and environment	3903

A.10.2.1.1.2	Test requirements	3906

A.10.2.2	UE timing advance	3907

A.10.2.2.1	UE Timing Advance Adjustment Accuracy with PSCell under DL CCA	3907

A.10.2.2.1.1	Test Purpose and Environment	3907

A.10.2.2.1.2	Test Parameters	3907

A.10.2.2.1.3	Test Requirements	3911

A.10.3	Signalling characteristics	3911

A.10.3.1	Radio link monitoring	3911

A.10.3.1.1	Introduction	3911

A.10.3.1.2	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	3912

A.10.3.1.2.1	Test purpose and environment	3912

A.10.3.1.2.2	Test requirements	3916

A.10.3.1.3	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	3917

A.10.3.1.3.1	Test purpose and environment	3917

A.10.3.1.3.2	Test requirements	3922

A.10.3.1.4	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode	3922

A.10.3.1.4.1	Test purpose and environment	3922

A.10.3.1.4.2	Test requirements	3922

A.10.3.1.5	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode	3922

A.10.3.1.5.1	Test purpose and environment	3922

A.10.3.1.5.2	Test requirements	3922

A.10.3.2	Interruption	3922

A.10.3.2.1	E-UTRAN – NR interruptions during SCell operations with CCA	3922

A.10.3.2.1.1	Test Purpose and Environment	3922

A.10.3.2.1.2	Test Requirements	3927

A.10.3.3	SCell activation and deactivation delay	3927

A.10.3.3.1	SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle	3927

A.10.3.3.1.1	Test Purpose and Environment	3927

A.10.3.3.1.2	Test Requirements	3932

A.10.3.3.2 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle	3932

A.10.3.3.2.1	Test Purpose and Environment	3932

A.10.3.3.2.2	Test Requirements	3933

A.10.3.3.3	SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA	3933

A.10.3.3.3.1	Test Purpose and Environment	3933

A.10.3.3.3.2	Test Requirements	3934

A.10.3.4	Beam failure detection and link recovery procedures	3934

A.10.3.4.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	3934

A.10.3.4.1.1	Test Purpose and Environment	3934

A.10.3.4.1.2	Test Requirements	3939

A.10.3.4.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	3939

A.10.3.4.2.1	Test Purpose and Environment	3939

A.10.3.4.2.2	Test Requirements	3945

A.10.3.5	Active BWP switching	3945

A.10.3.5.1	UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA in EN-DC	3945

A.10.3.5.1.2	Test Requirements	3950

A.10.3.5.2	DCI-based and Timer-based Active BWP Switch	3951

A.10.3.5.2.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	3951

A.10.3.5.2.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	3954

A.10.3.5.3	RRC-based Active BWP Switch	3958

A.10.3.5.3.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	3958

A.10.3.6	PSCell addition and release delay	3962

A.10.3.6.1	Addition and Release Delay of known NR PSCell on the carrier under CCA	3962

A.10.3.6.1.1	Test purpose and environment	3962

A.10.3.6.1.2	Test Requirements	3966

A.10.3.7	Void	3967

A.10.4	Measurement procedure	3967

A.10.4.1	Intra-frequency measurements	3967

A.10.4.1.1	Event-triggered reporting tests on PSCC without gaps under non-DRX	3967

A.10.4.1.1.1	Test purpose and environment	3967

A.10.4.1.1.2	Test parameters	3967

A.10.4.1.1.3	Test Requirements	3967

A.10.4.1.2	Event-triggered reporting tests on PSCC without gaps under DRX	3968

A.10.4.1.2.1	Test purpose and environment	3968

A.10.4.1.2.2	Test parameters	3968

A.10.4.1.2.3	Test Requirements	3968

A.10.4.1.3	Event-triggered reporting tests on PSCC with per-UE gaps under non-DRX	3968

A.10.4.1.3.1	Test purpose and environment	3968

A.10.4.1.3.2	Test parameters	3968

A.10.4.1.3.3	Test Requirements	3968

A.10.4.1.4	Event-triggered reporting tests on PSCC with per-UE gaps under DRX	3969

A.10.4.1.4.1	Test purpose and environment	3969

A.10.4.1.4.2	Test parameters	3969

A.10.4.1.4.3	Test Requirements	3969

A.10.4.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	3969

A.10.4.1.5.1	Test purpose and environment	3969

A.10.4.1.5.2	Test parameters	3969

A.10.4.1.5.3	Test Requirements	3969

A.10.4.1.6	Event-triggered reporting tests on SCC without gaps under DRX	3970

A.10.4.1.6.1	Test purpose and environment	3970

A.10.4.1.6.2	Test parameters	3970

A.10.4.1.6.3	Test Requirements	3970

A.10.4.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	3970

A.10.4.1.7.1	Test purpose and environment	3970

A.10.4.1.7.2	Test parameters	3970

A.10.4.1.7.3	Test Requirements	3970

A.10.4.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	3971

A.10.4.1.8.1	Test purpose and environment	3971

A.10.4.1.8.2	Test parameters	3971

A.10.4.1.8.3	Test Requirements	3971

A.10.4.1.9	RSSI measurement reporting on PSCC	3971

A.10.4.1.9.1	Test purpose and environment	3971

A.10.4.1.9.2	Test parameters	3971

A.10.4.1.10	Channel occupancy measurement reporting on PSCC	3972

A.10.4.1.10.1	Test purpose and environment	3972

A.10.4.1.10.2	Test parameters	3972

A.10.4.1.11	RSSI measurement reporting on SCC	3972

A.10.4.1.11.1	Test purpose and environment	3972

A.10.4.1.11.2	Test parameters	3972

A.10.4.1.12	Channel occupancy measurement reporting on SCC	3973

A.10.4.1.12.1	Test purpose and environment	3973

A.10.4.1.12.2	Test parameters	3973

A.10.4.2	Inter-frequency measurements	3973

A.10.4.2.1	RSSI measurement reporting	3973

A.10.4.2.1.1	Test purpose and environment	3973

A.10.4.2.1.2	Test parameters	3973

A.10.4.2.2	Channel occupancy measurement reporting	3974

A.10.4.2.2.1	Test purpose and environment	3974

A.10.4.2.2.2	Test parameters	3974

A.10.4.2.3	EN-DC event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used	3974

A.10.4.2.3.1	Test Purpose and Environment	3974

A.10.4.2.3.2	Test Requirements	3979

A.10.4.2.4	EN-DC event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used	3980

A.10.4.2.4.1	Test Purpose and Environment	3980

A.10.4.2.4.2	Test Requirements	3984

A.10.4.2.5	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used	3985

A.10.4.2.5.1	Test Purpose and Environment	3985

A.10.4.2.5.2	Test Requirements	3989

A.10.4.2.6	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used	3990

A.10.4.2.6.1	Test Purpose and Environment	3990

A.10.4.2.6.2	Test Requirements	3994

A.10.4.2.7	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	3995

A.10.4.2.7.1	Test Purpose and Environment	3995

A.10.4.2.7.2	Test Requirements	4001

A.10.4.2.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	4001

A.10.4.2.8.1	Test Purpose and Environment	4001

A.10.4.2.8.2	Test Requirements	4007

A.10.4.2.9	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	4008

A.10.4.2.9.1	Test Purpose and Environment	4008

A.10.4.2.9.2	Test Requirements	4013

A.10.4.2.10	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	4013

A.10.4.2.10.1	Test Purpose and Environment	4013

A.10.4.2.10.2	Test Requirements	4019

A.10.4.3	L1-RSRP measurements for beam reporting	4020

A.10.4.3.1	SSB based L1-RSRP measurement on PSCC when DRX is not used	4020

A.10.4.3.1.1	Test Purpose and Environment	4020

A.10.4.3.1.2	Test parameters	4020

A.10.4.3.1.3	Test Requirements	4022

A.10.4.3.2	SSB based L1-RSRP measurement on PSCC when DRX is used	4023

A.10.4.3.2.1	Test Purpose and Environment	4023

A.10.4.3.2.2	Test parameters	4023

A.10.4.3.2.3	Test Requirements	4025

A.10.4.3.3	SSB based L1-RSRP measurement on SCC when DRX is not used	4026

A.10.4.3.3.1	Test Purpose and Environment	4026

A.10.4.3.3.2	Test parameters	4026

A.10.4.3.3.3	Test Requirements	4029

A.10.4.3.4	SSB based L1-RSRP measurement on SCC when DRX is used	4030

A.10.4.3.4.1	Test Purpose and Environment	4030

A.10.4.3.4.2	Test parameters	4030

A.10.4.3.4.3	Test Requirements	4033

A.10.4.4	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	4034

A.10.4.4.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4034

A.10.4.4.1.1	Test Purpose and Environment	4034

A.10.4.4.1.2	Test Requirements	4038

A.10.4.4.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4039

A.10.4.4.2.1	Test Purpose and Environment	4039

A.10.4.4.2.2	Test Requirements	4044

A.10.4.4.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4045

A.10.4.4.3.1	Test Purpose and Environment	4045

A.10.4.4.3.2	Test Requirements	4049

A.10.4.4.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4050

A.10.4.4.4.1	Test Purpose and Environment	4050

A.10.4.4.4.2	Test Requirements	4055

A.10.5	Measurement performance	4056

A.10.5.1	SS-RSRP	4056

A.10.5.1.1	Intra-frequency measurement accuracy on a CCA serving cell	4056

A.10.5.1.1.1	Test Purpose and Environment	4056

A.10.5.1.1.2	Test parameters	4056

A.10.5.1.1.3	Test Requirements	4059

A.10.5.1.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4059

A.10.5.1.2.1	Test Purpose and Environment	4059

A.10.5.1.2.2	Test parameters	4059

A.10.5.1.2.3	Test Requirements	4062

A.10.5.2	SS-RSRQ	4062

A.10.5.2.1	Intra-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4062

A.10.5.2.1.1	Test Purpose and Environment	4062

A.10.5.2.1.2	Test Parameters	4062

A.10.5.2.1.3	Test Requirements	4065

A.10.5.2.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4065

A.10.5.2.2.1	Test Purpose and Environment	4065

A.10.5.2.2.2	Test Parameters	4065

A.10.5.2.2.3	Test Requirements	4068

A.10.5.3	SS-SINR	4068

A.10.5.3.1	Intra-frequency measurement accuracy on PSCC	4068

A.10.5.3.1.1	Test Purpose and Environment	4068

A.10.5.3.1.2	Test Parameters	4068

A.10.5.3.1.3	Test Requirements	4071

A.10.5.3.2	Inter-frequency measurement accuracy on PSCC	4071

A.10.5.3.2.1	Test Purpose and Environment	4071

A.10.5.3.2.2	Test Parameters	4071

A.10.5.3.2.3	Test Requirements	4074

A.10.5.3.3	Intra-frequency measurement accuracy on SCC	4074

A.10.5.3.3.1	Test Purpose and Environment	4074

A.10.5.3.3.2	Test Parameters	4074

A.10.5.3.3.3	Test Requirements	4077

A.10.5.4	L1-RSRP measurement for beam reporting with CCA serving cell	4077

A.10.5.4.1	SSB based L1-RSRP measurement	4077

A.10.5.4.1.1	Test Purpose and Environment	4077

A.10.5.4.1.2	Test parameters	4078

A.10.5.4.1.3	Test Requirements	4080

A.10.5.5	RSSI	4080

A.10.5.5.1 	RSSI measurement accuracy on PSCC with CCA	4080

A.10.5.5.1.1	Test Purpose and Environment	4080

A.10.5.5.1.2	Test parameters	4080

A.10.5.5.1.3	Test Requirements	4083

A.10.5.5.2 	RSSI measurement accuracy on SCC with CCA	4083

A.10.5.5.2.1	Test Purpose and Environment	4083

A.10.5.5.2.2	Test parameters	4083

A.10.5.5.2.3	Test Requirements	4086

A.10.5.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4086

A.10.5.5.3.1	Test Purpose and Environment	4086

A.10.5.5.3.2	Test parameters	4086

A.10.5.5.3.3	Test Requirements	4089

A.10.5.6	Channel occupancy	4089

A.10.5.6.1 	Channel occupancy measurement accuracy on PSCC with CCA	4089

A.10.5.6.1.1	Test Purpose and Environment	4089

A.10.5.6.1.2	Test parameters	4089

A.10.5.6.1.3	Test Requirements	4093

A.10.5.6.2 	Channel occupancy measurement accuracy on SCC with CCA	4093

A.10.5.6.2.1	Test Purpose and Environment	4093

A.10.5.6.2.2	Test parameters	4093

A.10.5.6.2.3	Test Requirements	4096

A.10.5.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4096

A.10.5.6.3.1	Test Purpose and Environment	4096

A.10.5.6.3.2	Test parameters	4096

A.10.5.6.3.3	Test Requirements	4100

A.11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	4100

A.11.1	RRC_IDLE state mobility	4100

A.11.1.1	Cell re-selection with both source and target NR carrier frequencies under CCA	4100

A.11.1.1.1	Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell	4100

A.11.1.1.1.1	Test Purpose and Environment	4100

A.11.1.1.1.2	Test Parameters	4100

A.11.1.1.1.3	Test Requirements	4103

A.11.1.1.2	Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell	4103

A.11.1.1.2.1	Test Purpose and Environment	4103

A.11.1.1.2.2	Test Parameters	4103

A.11.1.1.2.3	Test Requirements	4107

A.11.1.2	Cell re-selection to NR with source NR carrier frequency under CCA	4107

A.11.1.2.1	Cell reselection to FR1 inter-frequency NR case when serving cell is subject to CCA	4107

A.11.1.2.1.1	Test Purpose and Environment	4107

A.11.1.2.1.2	Test Parameters	4108

A.11.1.2.1.3	Test Requirements	4114

A.11.1.3	Cell re-selection from NR carrier with target NR carrier frequency under CCA	4115

A.11.1.3.1	Cell reselection to FR1 inter-frequency NR case when target cell is subject to CCA	4115

A.11.1.3.1.1	Test Purpose and Environment	4115

A.11.1.3.1.2	Test Parameters	4115

A.11.1.3.1.3	Test Requirements	4121

A.11.1.4	Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA	4122

A.11.1.4.1	Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA	4122

A.11.1.4.1.1	Test Purpose and Environment	4122

A.11.1.4.1.2	Test Parameters	4122

A.11.1.4.1.3	Test Requirements	4125

A.11.1.4.2	Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA	4126

A.11.1.4.2.1	Test Purpose and Environment	4126

A.11.1.4.2.2	Test Requirements	4129

A.11.2	RRC_CONNECTED state mobility	4130

A.11.2.1	Handover	4130

A.11.2.1.1	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell	4130

A.11.2.1.1.1	Test Purpose and Environment	4130

A.11.2.1.1.2	Test Parameters	4130

A.11.2.1.1.3 Test Requirements	4133

A.11.2.1.2	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	4133

A.11.2.1.2.1	Test Purpose and Environment	4133

A.11.2.1.2.2	Test Parameters	4133

A.11.2.1.2.3	Test Requirements	4136

A.11.2.1.3	Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	4136

A.11.2.1.3.1	Test Purpose and Environment	4136

A.11.2.1.3.2	Test Parameters	4136

A.11.2.1.3.3 Test Requirements	4139

A.11.2.1.4	Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell	4139

A.11.2.1.4.1	Test Purpose and Environment	4139

A.11.2.1.4.2	Test Parameters	4139

A.11.2.1.4.3 Test Requirements	4143

A.11.2.1.5	Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell	4144

A.11.2.1.5.1	Test Purpose and Environment	4144

A.11.2.1.5.2	Test Parameters	4144

A.11.2.1.5.3 Test Requirements	4147

A.11.2.1.6	Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell	4148

A.11.2.1.6.1	Test Purpose and Environment	4148

A.11.2.1.6.2	Test Parameters	4148

A.11.2.1.6.3 Test Requirements	4151

A.11.2.1.7	 SA NR FR1 carrier under CCA - E-UTRAN handover with known target cell	4152

A.11.2.1.7.1	Test Purpose and Environment	4152

A.11.2.1.7.2	Test Requirements	4158

A.11.2.1.8	SA NR FR1 carrier under CCA - E-UTRAN handover with unknown target cell	4158

A.11.2.1.8.1	Test Purpose and Environment	4158

A.11.2.1.8.2	Test Requirements	4163

A.11.2.1.9	Handover with PSCell from NR SA to EN-DC with known target PSCell using CCA	4163

A.11.2.1.9.1	Test Purpose and Environment	4163

A.11.2.1.9.2	Test Requirements	4172

A.11.2.2	RRC connection mobility control	4173

A.11.2.2.1	RRC re-establishment	4173

A.11.2.2.1.1	Intra-frequency RRC Re-establishment with CCA in FR1	4173

A.11.2.2.1.2	Inter-frequency RRC Re-establishment with CCA in FR1	4176

A.11.2.2.1.4	Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA	4183

A.11.2.2.2	Random Access	4188

A.11.2.2.2.1	4-step RA type contention-based random access for NR PCell with CCA	4188

A.11.2.2.2.1.1	Test Purpose and Environment	4188

A.11.2.2.2.1.2	Test Requirements	4191

A.11.2.2.2.1.2.1	Random Access Preamble Transmission	4191

A.11.2.2.2.1.2.2	Random Access Response Reception	4191

A.11.2.2.2.1.2.3	No Random Access Response Reception	4192

A.11.2.2.2.1.2.4	Receiving an UL grant for msg3 retransmission	4192

A.11.2.2.2.1.2.5	Reception of an Incorrect Message over Temporary C-RNTI	4192

A.11.2.2.2.1.2.6	Reception of a Correct Message over Temporary C-RNTI	4193

A.11.2.2.2.1.2.7	Contention Resolution Timer expiry	4193

A.11.2.2.2.2	4-step RA type non-contention based random access for NR PSCell with CCA	4193

A.11.2.2.2.2.1	Test Purpose and Environment	4193

A.11.2.2.2.2.2	Test Requirements	4196

A.11.2.2.2.2.2.1	SSB-based Random Access Preamble Transmission	4196

A.11.2.2.2.2.2.2	Random Access Response Reception	4197

A.11.2.2.2.2.2.3	No Random Access Response Reception	4197

A.11.2.2.2.3	2-step RA type contention-based random access for NR PCell with CCA	4197

A.11.2.2.2.3.1	Test Purpose and Environment	4197

A.11.2.2.2.3.2	Test Requirements	4201

A.11.2.2.2.3.2.1	MsgA Transmission	4201

A.11.2.2.2.3.2.2	MsgB Reception	4201

A.11.2.2.2.3.2.3	No MsgB Reception	4202

A.11.2.2.2.4	2-step RA type non-contention-based random access for NR PCell with CCA	4202

A.11.2.2.2.4.1	Test Purpose and Environment	4202

A.11.2.2.2.4.2	Test Requirements	4206

A.11.2.2.2.4.2.1	MsgA Transmission	4206

A.11.2.2.2.4.2.2	MsgB Reception	4207

A.11.2.2.2.4.2.3	No MsgB Reception	4207

A.11.2.2.3	RRC connection release with redirection	4208

A.11.2.2.3.1	Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA	4208

A.11.2.2.3.2	Redirection from NR FR1 carrier without CCA to NR FR1 carrier with CCA	4212

A.11.3	Timing	4217

A.11.3.1	UE transmit timing	4217

A.11.3.1.1	UE Transmit Timing Test with PCell under DL CCA	4217

A.11.3.1.1.1	Test Purpose and environment	4217

A.11.3.1.1.2	Test requirements	4220

A.11.3.2	UE timing advance	4221

A.11.3.2.1	UE Timing Advance Adjustment Accuracy with PCell under DL CCA	4221

A.11.3.2.1.1	Test Purpose and Environment	4221

A.11.3.2.1.2	Test Parameters	4221

A.11.3.2.1.3	Test Requirements	4225

A.11.4	Signalling characteristics	4225

A.11.4.1	Radio link monitoring	4225

A.11.4.1.1	Introduction	4225

A.11.4.1.2	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	4226

A.11.4.1.2.1	Test purpose and environment	4226

A.11.4.1.2.2	Test requirements	4230

A.11.4.1.3	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	4230

A.11.4.1.3.1	Test purpose and environment	4230

A.11.4.1.3.2	Test requirements	4236

A.11.4.1.4	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode	4236

A.11.4.1.4.1	Test purpose and environment	4236

A.11.4.1.4.2	Test requirements	4236

A.11.4.1.5	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode	4236

A.11.4.1.5.1	Test purpose and environment	4236

A.11.4.1.5.2	Test requirements	4236

A.11.4.2	Interruption	4237

A.11.4.2.1	NR interruptions during Scell operations with CCA on PCell and SCell	4237

A.11.4.2.1.1 Test Purpose and Environment	4237

A.11.4.2.1.2	Test Requirements	4241

A.11.4.3	SCell activation and deactivation delay	4241

A.11.4.3.1	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle	4241

A.11.4.3.1.1	Test Purpose and Environment	4241

A.11.4.3.1.2	Test Requirements	4246

A.11.4.3.2	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 640 ms SCell measurement cycle	4246

A.11.4.3.2.1	Test Purpose and Environment	4246

A.11.4.3.2.2	Test Requirements	4247

A.11.4.3.3	SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA	4247

A.11.4.3.3.1	Test Purpose and Environment	4247

A.11.4.3.3.2	Test Requirements	4248

A.11.4.4	Beam failure detection and link recovery procedures	4248

A.11.4.4.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	4248

A.11.4.4.1.1	Test Purpose and Environment	4248

A.11.4.4.1.2	Test Requirements	4253

A.11.4.4.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	4253

A.11.4.4.2.1	Test Purpose and Environment	4253

A.11.4.4.2.2	Test Requirements	4259

A.11.4.5	Active BWP switching	4259

A.11.4.5.1	UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA	4259

A.11.4.5.1.1	Test Purpose and Environment	4259

A.11.4.5.1.2	Test Requirements	4263

A.11.4.5.2	DCI-based and Timer-based Active BWP Switch	4264

A.11.4.5.2.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA	4264

A.11.4.5.2.2	NR FR1 DL active BWP switch with non-DRX in SA	4267

A.11.4.5.3	RRC-based Active BWP Switch	4270

A.11.4.5.3.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	4270

A.11.4.6	Void	4273

A.11.5	Measurement procedure	4273

A.11.5.1	Intra-frequency measurements	4273

A.11.5.1.1	Event-triggered reporting tests on PCC without gaps under non-DRX	4273

A.11.5.1.1.1	Test purpose and environment	4273

A.11.5.1.1.2	Test parameters	4273

A.11.5.1.1.3	Test Requirements	4274

A.11.5.1.2	Event-triggered reporting tests on PCC without gaps under DRX	4274

A.11.5.1.2.1	Test purpose and environment	4274

A.11.5.1.2.2	Test parameters	4274

A.11.5.1.2.3	Test Requirements	4274

A.11.5.1.3	Event-triggered reporting tests on PCC with per-UE gaps under non-DRX	4275

A.11.5.1.3.1	Test purpose and environment	4275

A.11.5.1.3.2	Test parameters	4275

A.11.5.1.3.3	Test Requirements	4275

A.11.5.1.4	Event-triggered reporting tests on PCC with per-UE gaps under DRX	4275

A.11.5.1.4.1	Test purpose and environment	4275

A.11.5.1.4.2	Test parameters	4275

A.11.5.1.4.3	Test Requirements	4275

A.11.5.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	4276

A.11.5.1.5.1	Test purpose and environment	4276

A.11.5.1.5.2	Test parameters	4276

A.11.5.1.6	Event-triggered reporting tests on SCC without gaps under DRX	4276

A.11.5.1.6.1	Test purpose and environment	4276

A.11.5.1.6.2	Test parameters	4276

A.11.5.1.6.3	Test Requirements	4276

A.11.5.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	4277

A.11.5.1.7.1	Test purpose and environment	4277

A.11.5.1.7.2	Test parameters	4277

A.11.5.1.7.3	Test Requirements	4277

A.11.5.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	4277

A.11.5.1.8.1	Test purpose and environment	4277

A.11.5.1.8.2	Test parameters	4277

A.11.5.1.8.3	Test Requirements	4277

A.11.5.1.9	RSSI measurement reporting on PCC	4278

A.11.5.1.9.1	Test purpose and environment	4278

A.11.5.1.9.2	Test parameters	4278

A.11.5.1.10	Channel occupancy measurement reporting on PCC	4278

A.11.5.1.10.1	Test purpose and environment	4278

A.11.5.1.10.2	Test parameters	4278

A.11.5.1.11	RSSI measurement reporting on SCC	4278

A.11.5.1.11.1	Test purpose and environment	4278

A.11.5.1.11.2	Test parameters	4279

A.11.5.1.12	Channel occupancy measurement reporting on SCC	4279

A.11.5.1.12.1	Test purpose and environment	4279

A.11.5.1.12.2	Test parameters	4279

A.11.5.2	Inter-frequency measurements	4279

A.11.5.2.1	RSSI measurement reporting	4279

A.11.5.2.1.1	Test purpose and environment	4279

A.11.5.2.1.2	Test parameters	4280

A.11.5.2.2	Channel occupancy measurement reporting	4280

A.11.5.2.2.1	Test purpose and environment	4280

A.11.5.2.2.2	Test parameters	4280

A.11.5.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	4280

A.11.5.2.3.1	Test Purpose and Environment	4280

A.11.5.2.3.2	Test Requirements	4284

A.11.5.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	4285

A.11.5.2.4.1	Test Purpose and Environment	4285

A.11.5.2.4.2	Test Requirements	4290

A.11.5.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	4290

A.11.5.2.5.1	Test Purpose and Environment	4290

A.11.5.2.5.2	Test Requirements	4294

A.11.5.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	4295

A.11.5.2.6.1	Test Purpose and Environment	4295

A.11.5.2.6.2	Test Requirements	4300

A.11.5.2.7	Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4300

A.11.5.2.7.1	Test Purpose and Environment	4300

A.11.5.2.7.2	Test Requirements	4304

A.11.5.2.8	Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4305

A.11.5.2.8.1	Test Purpose and Environment	4305

A.11.5.2.8.2	Test Requirements	4310

A.11.5.2.9	Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4310

A.11.5.2.9.1	Test Purpose and Environment	4310

A.11.5.2.9.2	Test Requirements	4315

A.11.5.2.10	Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4316

A.11.5.2.10.1	Test Purpose and Environment	4316

A.11.5.2.10.2	Test Requirements	4321

A.11.5.3	Inter-RAT E-UTRAN measurements	4322

A.11.5.4	L1-RSRP measurements for beam reporting	4322

A.11.5.4.1	SSB based L1-RSRP measurement when DRX is not used	4322

A.11.5.4.1.1	Test Purpose and Environment	4322

A.11.5.4.1.2	Test parameters	4322

A.11.5.4.1.3	Test Requirements	4324

A.11.5.4.2	SSB based L1-RSRP measurement when DRX is used	4325

A.11.5.4.2.1	Test Purpose and Environment	4325

A.11.5.4.2.2	Test parameters	4325

A.11.5.4.2.3	Test Requirements	4327

A.11.5.4.3	SSB based L1-RSRP measurement on SCC when DRX is not used	4328

A.11.5.4.3.1	Test Purpose and Environment	4328

A.11.5.4.3.2	Test parameters	4328

A.11.5.4.3.3	Test Requirements	4331

A.11.5.4.4	SSB based L1-RSRP measurement on SCC when DRX is used	4332

A.11.5.4.4.1	Test Purpose and Environment	4332

A.11.5.4.4.2	Test parameters	4332

A.11.5.4.4.3	Test Requirements	4335

A.11.6	Measurement performance	4336

A.11.6.1	SS-RSRP	4336

A.11.6.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	4336

A.11.6.1.1.1	Test Purpose and Environment	4336

A.11.6.1.1.2	Test parameters	4336

A.11.6.1.1.3	Test Requirements	4338

A.11.6.1.2	Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	4338

A.11.6.1.2.1	Test Purpose and Environment	4338

A.11.6.1.2.2	Test parameters	4338

A.11.6.1.2.3	Test Requirements	4340

A.11.6.2	SS-RSRQ	4340

A.11.6.2.1	Intra-frequency measurement accuracy	4340

A.11.6.2.1.1	Test Purpose and Environment	4340

A.11.6.2.1.2	Test Parameters	4340

A.11.6.2.1.3	Test Requirements	4343

A.11.6.2.2	Inter-frequency measurement accuracy	4343

A.11.6.2.2.1	Test Purpose and Environment	4343

A.11.6.2.2.2	Test Parameters	4343

A.11.6.2.2.3	Test Requirements	4346

A.11.6.2.3	Intra-frequency measurement accuracy on SCC	4346

A.11.6.2.3.1	Test Purpose and Environment	4346

A.11.6.2.3.2	Test Parameters	4346

A.11.6.2.3.3	Test Requirements	4349

A.11.6.2.4	Inter-frequency measurement accuracy	4349

A.11.6.2.4.1	Test Purpose and Environment	4349

A.11.6.2.4.2	Test Parameters	4349

A.11.6.2.4.3	Test Requirements	4357

A.11.6.3	SS-SINR	4358

A.11.6.3.1	Intra-frequency measurement accuracy	4358

A.11.6.3.1.1	Test Purpose and Environment	4358

A.11.6.3.1.2	Test Parameters	4358

A.11.6.3.1.3	Test Requirements	4361

A.11.6.3.2	Inter-frequency measurement accuracy	4361

A.11.6.3.2.1	Test Purpose and Environment	4361

A.11.6.3.2.2	Test Parameters	4361

A.11.6.3.2.3	Test Requirements	4364

A.11.6.3.3	Intra-frequency measurement accuracy on SCC	4364

A.11.6.3.3.1	Test Purpose and Environment	4364

A.11.6.3.3.2	Test Parameters	4364

A.11.6.3.3.3	Test Requirements	4367

A.11.6.3.4	Inter-frequency measurement accuracy	4367

A.11.6.3.4.1	Test Purpose and Environment	4367

A.11.6.3.4.2	Test Parameters	4367

A.11.6.3.4.3	Test Requirements	4375

A.11.6.4	L1-RSRP measurement for beam reporting with CCA serving cell	4375

A.11.6.4.1	SSB based L1-RSRP measurement	4375

A.11.6.4.1.1	Test Purpose and Environment	4375

A.11.6.4.1.2	Test parameters	4376

A.11.6.4.1.3	Test Requirements	4379

A.11.6.5	RSSI	4379

A.11.6.5.1 	Intra-frequency RSSI measurement accuracy on PCC with CCA	4379

A.11.6.5.1.1	Test Purpose and Environment	4379

A.11.6.5.1.2	Test parameters	4379

A.11.6.5.1.3	Test Requirements	4382

A.11.6.5.2 	Intra-frequency RSSI measurement accuracy on SCC with CCA	4382

A.11.6.5.2.1	Test Purpose and Environment	4382

A.11.6.5.2.2	Test parameters	4382

A.11.6.5.2.3	Test Requirements	4385

A.11.6.5.3	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4385

A.11.6.5.3.1	Test Purpose and Environment	4385

A.11.6.5.3.2	Test parameters	4385

A.11.6.5.3.3	Test Requirements	4388

A.11.6.6	Channel occupancy	4388

A.11.6.6.1	Intra-frequency channel occupancy measurement accuracy on PCC with CCA	4388

A.11.6.6.1.1	Test Purpose and Environment	4388

A.11.6.6.1.2	Test parameters	4388

A.11.6.6.1.3	Test Requirements	4391

A.11.6.6.2	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	4391

A.11.6.6.2.1	Test Purpose and Environment	4391

A.11.6.6.2.2	Test parameters	4391

A.11.6.6.2.3	Test Requirements	4394

A.11.6.6.3	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4394

A.11.6.6.3.1	Test Purpose and Environment	4394

A.11.6.6.3.2	Test parameters	4394

A.11.6.6.3.3	Test Requirements	4397

A.11.6.7	E-UTRAN RSRP	4397

A.11.6.8	E-UTRAN RSRQ	4397

A.11.6.9	E-UTRAN SINR	4397

A.12	E-UTRA Standalone Tests with at Least One NR Cell under CCA	4397

A.12.1	RRC_IDLE state mobility	4397

A.12.1.1	Inter-RAT cell re-selection to NR on a carrier frequency with CCA	4397

A.12.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA	4397

A.12.1.1.1.1	Test Purpose and Environment	4397

A.12.1.1.1.2	Test Requirements	4402

A.12.2	RRC_CONNECTED state mobility	4403

A.12.2.1	Handover	4403

A.12.2.1.1	E-UTRAN - NR with CCA handover	4403

A.12.2.1.1.1	Test Purpose and Environment	4403

A.12.2.1.1.2	Test Requirements	4408

A.12.3	Signalling characteristics	4409

A.12.3.1	Interruptions	4409

A.12.4	Measurement procedure	4409

A.12.4.1	E-UTRANNR inter-RAT SFTD measurements	4409

A.12.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay with NR under CCA in non-DRX	4409

A.12.4.1.1.1	Test Purpose and Environment	4409

A.12.4.1.1.2	Test Requirements	4413

A.12.4.2	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	4413

A.12.4.2.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4413

A.12.4.2.1.1	Test Purpose and Environment	4413

A.12.4.2.1.2	Test Requirements	4417

A.12.4.2.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4418

A.12.4.2.2.1	Test Purpose and Environment	4418

A.12.4.2.2.2	Test Requirements	4422

A.12.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4422

A.12.4.2.3.1	Test Purpose and Environment	4422

A.12.4.2.3.2	Test Requirements	4426

A.12.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4427

A.12.4.2.4.1	Test Purpose and Environment	4427

A.12.4.2.4.2	Test Requirements	4431

A.12.4.2.5	RSSI measurement reporting	4431

A.12.4.2.5.1	Test purpose and environment	4431

A.12.4.2.5.2	Test parameters	4431

A.12.4.2.6	Channel occupancy measurement reporting	4432

A.12.4.2.6.1	Test purpose and environment	4432

A.12.4.2.6.2	Test parameters	4432

A.12.5	Measurement performance	4432

A.12.5.1	E-UTRANNR SFTD	4432

A.12.5.1.1	Inter-RAT SFTD accuracy with NR target cell under CCA	4432

A.12.5.1.1.1	Test Purpose	4432

A.12.5.1.1.2	Test Environment	4433

A.12.5.1.1.3	Test Requirements	4438

A.12.5.2	E-UTRANNR SS-RSRP	4438

A.12.5.3	E-UTRANNR SS-RSRQ	4438

A.12.5.4	E-UTRANNR SS-SINR	4438

A.12.5.5	E-UTRANNR RSSI	4438

A.12.5.6	E-UTRANNR channel occupancy	4438

A.13	NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	4442

A.13.1	Timing	4442

A.13.1.1	UE transmit timing	4442

A.13.1.2	Timing advance	4442

A.13.2	Signalling characteristics	4442

A.13.2.1	Interruption	4442

A.13.2.1.1	NR interruptions during SCell operations with CCA on SCell	4442

A.13.2.1.1.1	Test Purpose and Environment	4442

A.13.2.1.1.2	Test Requirements	4446

A.13.2.2	SCell activation and deactivation delay	4446

A.13.2.2.1	SCell Activation and Deactivation of known SCell under CCA, 160 ms SCell measurement cycle	4446

A.13.2.2.1.1	Test Purpose and Environment	4446

A.13.2.2.1.2	Test Requirements	4451

A.13.2.2.2 SCell Activation and Deactivation of known SCell under CCA, 640 ms SCell measurement cycle	4451

A.13.2.2.2.1	Test Purpose and Environment	4451

A.13.2.2.2.2	Test Requirements	4452

A.13.2.2.3	SCell Activation and Deactivation of unknown SCell under CCA	4452

A.13.2.2.3.1	Test Purpose and Environment	4452

A.13.2.2.3.2	Test Requirements	4453

A.13.2.3	Void	4453

A.13.3	Measurement procedure	4453

A.13.3.1	Intra-frequency measurements	4453

A.13.3.1.1	Event-triggered reporting tests on SCC without gaps under non-DRX	4453

A.13.3.1.1.1	Test purpose and environment	4453

A.13.3.1.1.2	Test parameters	4453

A.13.3.1.1.3	Test Requirements	4458

A.13.3.1.2	Event-triggered reporting tests on SCC without gaps under DRX	4458

A.13.3.1.2.1	Test purpose and environment	4458

A.13.3.1.2.2	Test parameters	4458

A.13.3.1.2.3	Test Requirements	4463

A.13.3.1.3	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	4463

A.13.3.1.3.1	Test purpose and environment	4463

A.13.3.1.3.2	Test parameters	4463

A.13.3.1.3.3	Test Requirements	4468

A.13.3.1.4	Event-triggered reporting tests on SCC with per-UE gaps under DRX	4468

A.13.3.1.4.1	Test purpose and environment	4468

A.13.3.1.4.2	Test parameters	4468

A.13.3.1.4.3	Test Requirements	4473

A.13.3.1.5	RSSI measurement reporting on SCC	4473

A.13.3.1.5.1	Test purpose and environment	4473

A.13.3.1.5.2	Test parameters	4473

A.13.3.1.6	Channel occupancy measurement reporting on SCC	4474

A.13.3.1.6.1	Test purpose and environment	4474

A.13.3.1.6.2	Test parameters	4475

A.13.3.2	Inter-frequency measurements	4475

A.13.3.2.1	RSSI measurement reporting	4475

A.13.3.2.1.1	Test purpose and environment	4475

A.13.3.2.1.2	Test parameters	4475

A.13.3.2.2	Channel occupancy measurement reporting	4476

A.13.3.2.2.1	Test purpose and environment	4476

A.13.3.2.2.2	Test parameters	4476

A.13.3.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	4476

A.13.3.2.3.1	Test Purpose and Environment	4476

A.13.3.2.3.2	Test Requirements	4481

A.13.3.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	4481

A.13.3.2.4.1	Test Purpose and Environment	4481

A.13.3.2.4.2	Test Requirements	4486

A.13.3.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	4487

A.13.3.2.5.1	Test Purpose and Environment	4487

A.13.3.2.5.2	Test Requirements	4492

A.13.3.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	4492

A.13.3.2.6.1	Test Purpose and Environment	4492

A.13.3.2.6.2	Test Requirements	4498

A.13.3.3	L1-RSRP measurements for beam reporting	4499

A.13.3.3.1	SSB based L1-RSRP measurement when DRX is not used	4499

A.13.3.3.1.1	Test Purpose and Environment	4499

A.13.3.3.1.2	Test parameters	4499

A.13.3.3.1.3	Test Requirements	4503

A.13.3.3.2	SSB based L1-RSRP measurement when DRX is used	4503

A.13.3.3.2.1	Test Purpose and Environment	4503

A.13.3.3.2.2	Test parameters	4503

A.13.3.3.2.3	Test Requirements	4507

A.13.4	Measurement performance	4507

A.13.4.1	SS-RSRP	4507

A.13.4.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	4507

A.13.4.1.1.1	Test Purpose and Environment	4507

A.13.4.1.1.2	Test parameters	4507

A.13.4.1.1.3	Test Requirements	4509

A.13.4.2	SS-RSRQ	4509

A.13.4.2.1	Intra-frequency measurement accuracy on SCC	4509

A.13.4.2.1.1	Test Purpose and Environment	4509

A.13.4.2.1.2	Test Parameters	4509

A.13.4.2.1.3	Test Requirements	4516

A.13.4.3	SS-SINR	4517

A.13.4.3.1	Intra-frequency measurement accuracy on SCC	4517

A.13.4.3.1.1	Test Purpose and Environment	4517

A.13.4.3.1.2	Test Parameters	4517

A.13.4.3.1.3	Test Requirements	4524

A.13.4.4	L1-RSRP measurement for beam reporting with CCA serving cell	4524

A.13.4.4.1	SSB based L1-RSRP measurement	4524

A.13.4.4.1.1	Test Purpose and Environment	4524

A.13.4.4.1.2	Test parameters	4524

A.13.4.4.1.3	Test Requirements	4528

A.13.4.5	RSSI	4528

A.13.4.5.1 	Intra-frequency RSSI measurement accuracy on a carrier with CCA	4528

A.13.4.5.1.1	Test Purpose and Environment	4528

A.13.4.5.1.2	Test parameters	4528

A.13.4.5.1.3	Test Requirements	4531

A.13.4.5.2	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4531

A.13.4.5.2.1	Test Purpose and Environment	4531

A.13.4.5.2.2	Test parameters	4531

A.13.4.5.2.3	Test Requirements	4535

A.13.4.6	Channel occupancy	4535

A.13.4.6.1	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	4535

A.13.4.6.1.1	Test Purpose and Environment	4535

A.13.4.6.1.2	Test parameters	4535

A.13.4.6.1.3	Test Requirements	4539

A.13.4.6.2	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4539

A.13.4.6.2.1	Test Purpose and Environment	4539

A.13.4.6.2.2	Test parameters	4539

A.13.4.6.2.3	Test Requirements	4543

A.14	NR standalone tests for Satellite access	4543

A.14.1	RRC_IDLE state mobility	4543

A.14.1.1	Cell reselection to FR1 intra-frequency NR case	4543

A.14.1.1.1	Test Purpose and Environment	4543

A.14.1.1.2	Test Parameters	4543

A.14.1.1.3	Test Requirements	4545

A.14.1.2	Cell reselection to FR1 intra-frequency NR cell for UE configured with [capability for enhanced requirements]	4546

A.14.1.2.1	Test Purpose and Environment	4546

A.14.1.2.2	Test Parameters	4546

A.14.1.2.3	Test Requirements	4550

A.14.1.3	Time-based cell reselection to FR1 intra-frequency NR cell	4550

A.14.1.3.1	Test Purpose and Environment	4550

A.14.1.3.2	Test Parameters	4550

A.14.1.3.3	Test Requirements	4552

A.14.1.4	Location-based cell reselection to FR1 intra-frequency NR cell	4553

A.14.1.4.1	Test Purpose and Environment	4553

A.14.1.4.2	Test Parameters	4553

A.14.1.4.3	Test Requirements	4555

A.14.1.5	Cell reselection to FR1 inter-frequency NR case	4556

A.14.1.5.1	Test Purpose and Environment	4556

A.14.1.5.2	Test Parameters	4556

A.14.1.5.3	Test Requirements	4558

A.14.1.6	Cell reselection to FR1 inter-frequency NR cell for UE configured with [capability for enhanced requirements]	4559

A.14.1.6.1	Test Purpose and Environment	4559

A.14.1.6.2	Test Parameters	4559

A.14.1.6.3	Test Requirements	4561

A.14.1.7	Time-based Cell reselection to FR1 inter-frequency NR satellite access case	4562

A.14.1.7.1	Test Purpose and Environment	4562

A.14.1.7.2	Test Parameters	4562

A.14.1.7.3	Test Requirements	4563

A.14.1.8	Location-based Cell reselection to FR1 inter-frequency NR satellite access case	4564

A.14.1.8.1	Test Purpose and Environment	4564

A.14.1.8.2	Test Parameters	4564

A.14.1.8.3	Test Requirements	4566

A.14.1.9	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	4567

A.14.1.9.1	Test Purpose and Environment	4567

A.14.1.9.2	Test Parameters	4567

A.14.1.9.3	Test Requirements	4570

A.14.1.10	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	4570

A.14.1.10.1	Test Purpose and Environment	4570

A.14.1.10.2	Test Parameters	4570

A.14.1.10.3	Test Requirements	4572

A.14.2	RRC_CONNECTED state mobility	4573

A.14.2.1	Handover	4573

A.14.2.1.1	Intra-frequency SAN Handover from FR1 to FR1	4573

A.14.2.1.1.1	Test Purpose and Environment	4573

A.14.2.1.1.2	Test Parameters	4573

A.14.2.1.1.3	Test Requirements	4575

A.14.2.1.2	Inter-frequency SAN Handover from FR1 to FR1	4576

A.14.2.1.2.1	Test Purpose and Environment	4576

A.14.2.1.2.2	Test Parameters	4576

A.14.2.1.2.3	Test Requirements	4578

A.14.2.1.3	Intra-frequency SAN time-based conditional Handover from FR1 to FR1	4578

A.14.2.1.3.1	Test Purpose and Environment	4578

A.14.2.1.3.2	Test Parameters	4578

A.14.2.1.3.3	Test Requirements	4582

A.14.2.1.4	Inter-frequency SAN time-based conditional Handover from FR1 to FR1	4582

A.14.2.1.4.1	Test Purpose and Environment	4582

A.14.2.1.4.2	Test Parameters	4582

A.14.2.1.4.3	Test Requirements	4584

A.14.2.1.5	Intra-frequency SAN distance-based conditional Handover from FR1 to FR1	4584

A.14.2.1.5.1	Test Purpose and Environment	4584

A.14.2.1.5.2	Test Parameters	4585

A.14.2.1.5.3	Test Requirements	4588

A.14.2.1.6	Inter-frequency SAN distance-based conditional Handover from FR1 to FR1	4588

A.14.2.1.6.1	Test Purpose and Environment	4588

A.14.2.1.6.2	Test Parameters	4588

A.14.2.1.6.3	Test Requirements	4592

A.14.2.2	RRC Connection Mobility Control	4592

A.14.2.2.1	SA: RRC Re-establishment for SAN	4592

A.14.2.2.1.1	Intra-frequency RRC Re-establishment in FR1	4592

A.14.2.2.1.2	Inter-frequency RRC Re-establishment in FR1	4595

A.14.2.2.2	Random Access	4598

A.14.2.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	4598

A.14.2.2.2.1.1	Test Purpose and Environment	4598

A.14.2.2.2.1.2	Test Requirements	4601

A.14.2.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	4602

A.14.2.2.2.2.1	Test Purpose and Environment	4602

A.14.2.2.2.2.2	Test Requirements	4606

A.14.2.2.3	RRC Connection Release with Redirection	4607

A.14.2.2.3.1	Redirection from NR in FR1 to NR in FR1	4607

A.14.2.2.3.1.1	Test Purpose and Environment	4607

A.14.2.2.3.1.2	Test Parameters	4607

A.14.2.2.3.1.3	Test Requirements	4610

A.14.3	Timing for Satellite Access	4610

A.14.3.1	UE transmit timing for Satellite Access	4610

A.14.3.1.1	NR UE Transmit Timing Test for FR1	4610

A.14.3.1.1.1	Test Purpose and environment	4610

A.14.3.1.1.2	Test requirements	4612

A.14.3.2	Timing advance for satellite access	4613

A.14.3.2.1	SA FR1 timing advance adjustment accuracy	4613

A.14.3.2.1.1	Test Purpose and Environment	4613

A.14.3.2.1.2	Test Parameters	4613

A.14.3.2.1.3	Test Requirements	4616

A.14.4	Signalling characteristics	4616

A.14.4.1	Radio link Monitoring	4616

A.14.4.1.1	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode	4616

A.14.4.1.1.1	Test Purpose and Environment	4616

A.14.4.1.1.2	Test Requirements	4621

A.14.4.1.2	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode	4621

A.14.4.1.2.1	Test Purpose and Environment	4621

A.14.4.1.2.2	Test Requirements	4627

A.14.4.1.3	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode	4627

A.14.4.1.3.1	Test Purpose and Environment	4627

A.14.4.1.3.2	Test Requirements	4631

A.14.4.1.4	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode	4631

A.14.4.1.4.1	Test Purpose and Environment	4631

A.14.4.1.4.2	Test Requirements	4636

A.14.4.1.5	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode	4636

A.14.4.1.5.1	Test Purpose and Environment	4636

A.14.4.1.5.2	Test Requirements	4641

A.14.4.1.6	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode	4641

A.14.4.1.6.1	Test Purpose and Environment	4641

A.14.4.1.6.2	Test Requirements	4646

A.14.4.1.7	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode	4646

A.14.4.1.7.1	Test Purpose and Environment	4646

A.14.4.1.7.2	Test Requirements	4651

A.14.4.1.8	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode	4651

A.14.4.1.8.1	Test Purpose and Environment	4651

A.14.4.1.8.2	Test Requirements	4656

A.14.4.2	Beam Failure Detection and Link recovery procedures for satellite access	4656

A.14.4.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in non-DRX mode	4656

A.14.4.2.1.1	Test Purpose and Environment	4656

A.14.4.2.1.2	Test Requirements	4662

A.14.4.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in DRX mode	4662

A.14.4.2.2.1	Test Purpose and Environment	4662

A.14.4.2.2.2	Test Requirements	4667

A.14.4.2.3	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in non-DRX mode	4667

A.14.4.2.3.1	Test Purpose and Environment	4667

A.14.4.2.3.2	Test Requirements	4673

A.14.4.2.4	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in DRX mode	4673

A.14.4.2.4.1	Test Purpose and Environment	4673

A.14.4.2.4.2	Test Requirements	4678

A.14.4.2.5	Beam Failure Detection and Link Recovery Test for FR1 SCell for satellite access configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	4678

A.14.4.2.5.1	Test Purpose and Environment	4678

A.14.4.2.5.2	Test Requirements	4683

A.14.4.2.6	Beam Failure Detection and Link Recovery Test for FR1 SCell for satellite access configured with CSI-RS-based BFD and SSB-based LR in DRX mode	4683

A.14.4.2.6.1	Test Purpose and Environment	4683

A.14.4.2.6.2	Test Requirements	4688

A.14.4.3	Active BWP switch for satellite access	4688

A.14.4.3.1	DCI-based and Timer-based Active BWP Switch	4688

A.14.4.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA	4688

A.14.4.3.1.1.1	Test Purpose and Environment	4688

A.14.4.3.1.1.2	Test Requirements	4693

A.14.4.3.2	RRC-based Active BWP Switch	4693

A.14.4.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	4693

A.14.4.3.2.1.1	Test Purpose and Environment	4693

A.14.4.3.2.1.2	Test Requirements	4697

A.14.4.4	UE specific CBW change for satellite access	4697

A.14.4.4.1	UE specific CBW change on PCell in FR1 in non-DRX	4697

A.14.4.4.1.1	Test Purpose and Environment	4697

A.14.4.4.1.2	Test Requirements	4701

A.14.4.5	Pathloss reference signal switching delay	4701

A.14.4.5.1	MAC-CE based pathloss reference signal switch delay	4701

A.14.4.5.1.1	Test Purpose and Environment	4701

A.14.4.5.1.2	Test Requirements	4703

A.14.5	Measurement procedure	4704

A.14.5.1	Intra-frequency Measurements	4704

A.14.5.1.1	SA event triggered reporting tests without gap under non-DRX	4704

A.14.5.1.1.1	Test purpose and Environment	4704

A.14.5.1.1.2	Test parameters	4704

A.14.5.1.1.3	Test Requirements	4706

A.14.5.1.2	SA event triggered reporting tests without gap under DRX	4707

A.14.5.1.2.1	Test purpose and Environment	4707

A.14.5.1.2.2	Test parameters	4707

A.14.5.1.2.3	Test Requirements	4709

A.14.5.1.3	SA event triggered reporting tests without gap under non-DRX with SSB index reading	4710

A.14.5.1.3.1	Test purpose and Environment	4710

A.14.5.1.3.2	Test parameters	4710

A.14.5.1.3.3	Test Requirements	4712

A.14.5.1.4	SA event triggered reporting tests with single measurement gap under non-DRX for satellite access	4713

A.14.5.1.4.1	Test purpose and Environment	4713

A.14.5.1.4.2	Test parameters	4713

A.14.5.1.4.3	Test Requirements	4715

A.14.5.1.5	SA event triggered reporting tests with FNO concurrent gaps under DRX for satellite access	4716

A.14.5.1.5.1	Test purpose and Environment	4716

A.14.5.1.5.2	Test parameters	4716

A.15.5.1.5.3	Test Requirements	4718

A.14.5.1.6	SA event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for satellite access	4719

A.14.5.1.6.1	Test purpose and Environment	4719

A.14.5.1.6.2	Test parameters	4719

A.14.5.1.6.3	Test Requirements	4721

A.14.5.2	Inter-frequency Measurements	4722

A.14.5.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with single gap for satellite access	4722

A.14.5.2.1.1	Test Purpose and Environment	4722

A.14.5.2.1.2	Test Requirements	4726

A.14.5.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used with single gap for satellite access	4726

A.14.5.2.2.1	Test Purpose and Environment	4726

A.14.5.2.2.2	Test Requirements	4730

A.14.5.2.3	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used with single gap for satellite access	4730

A.14.5.2.3.1	Test Purpose and Environment	4730

A.14.5.2.3.2	Test Requirements	4734

A.14.5.2.4	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in fully non-overlapped for satellite access	4734

A.14.5.2.4.1	Test Purpose and Environment	4734

A.14.5.2.4.2	Test Requirements	4738

A.14.5.2.5	void	4738

A.14.5.2.5.1	void	4738

A.14.5.2.5.2	void	4738

A.14.5.2.6	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in partially partial overalpping for satellite access	4738

A.14.5.2.6.1	Test Purpose and Environment	4738

A.14.5.2.6.2	Test Requirements	4742

A.14.5.2.7	Event triggered reporting test without gap under non-DRX	4742

A.14.5.2.7.1	Test purpose and Environment	4742

A.14.5.2.7.2	Test parameters	4742

A.14.5.2.7.3	Test Requirements	4744

A.14.5.2.8	Event triggered reporting tests without gap under DRX	4745

A.14.5.2.8.1	Test purpose and Environment	4745

A.14.5.2.8.2	Test parameters	4745

A.14.5.2.8.3	Test Requirements	4747

A.14.5.3	L1-RSRP measurement for beam reporting for satellite access	4748

A.14.5.3.1	SSB based L1-RSRP measurement for satellite access when DRX is not used	4748

A.14.5.3.1.1	Test Purpose and Environment	4748

A.14.5.3.1.2	Test parameters	4748

A.14.5.3.1.3	Test Requirements	4751

A.14.5.3.2	SSB based L1-RSRP measurement for satellite access when DRX is used	4751

A.14.5.3.2.1	Test Purpose and Environment	4751

A.14.5.3.2.2	Test parameters	4751

A.14.5.3.2.3	Test Requirements	4755

A.14.5.3.3	CSI-RS based L1-RSRP measurement for satellite access when DRX is not used	4755

A.14.5.3.3.1	Test Purpose and Environment	4755

A.14.5.3.3.2	Test parameters	4755

A.14.5.3.3.3	Test Requirements	4759

A.14.5.3.4	CSI-RS based L1-RSRP measurement for satellite access when DRX is used	4759

A.14.5.3.4.1	Test Purpose and Environment	4759

A.14.5.3.4.2	Test parameters	4760

A.14.5.3.4.3	Test Requirements	4763

A.14.6	Measurement Performance requirements	4763

A.14.6.1	 SS-RSRP for SAN	4763

A.14.6.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	4763

A.14.6.1.1.1	Test Purpose and Environment	4763

A.14.6.1.1.2	Test parameters	4763

A.14.6.1.1.3	Test Requirements	4767

A.14.6.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	4767

A.14.6.1.2.1	Test Purpose and Environment	4767

A.14.6.1.2.2	Test parameters	4767

A.14.6.1.2.3	Test Requirements	4770

A.14.6.2	SS-RSRQ	4770

A.14.6.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for satellite access	4770

A.14.6.2.1.1	Test Purpose and Environment	4770

A.14.6.2.1.2	Test Parameters	4770

A.14.6.2.1.3	Test Requirements	4773

A.14.6.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for satellite access	4773

A.14.6.2.2.1	Test Purpose and Environment	4773

A.14.6.2.2.2	Test Parameters	4773

A.14.6.2.2.3	Test Requirements	4776

A.14.6.3	SS-SINR	4776

A.14.6.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	4776

A.14.6.3.1.1	Test Purpose and Environment	4776

A.14.6.3.1.2	Test Parameters	4776

A.14.6.3.1.3	Test Requirements	4780

A.14.6.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	4780

A.14.6.3.2.1	Test Purpose and Environment	4780

A.14.6.3.2.2	Test Parameters	4780

A.14.6.3.2.3	Test Requirements	4784

A.14.6.4	L1-RSRP measurement for beam reporting	4784

A.14.6.4.1	SSB based L1-RSRP measurement	4784

A.14.6.4.1.1	Test Purpose and Environment	4784

A.14.6.4.1.2	Test parameters	4784

A.14.6.4.1.3	Test Requirements	4787

A.14.6.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	4788

A.14.6.4.2.1	Test Purpose and Environment	4788

A.14.6.4.2.2	Test parameters	4788

A.14.6.4.2.3	Test Requirements	4791

A.15	NR standalone tests with one or more NR cells in FR2-2	4792

A.15.1	SA: RRC_IDLE state mobility	4792

A.15.1.1	Cell re-selection to NR	4792

A.15.1.1.1	Cell reselection to FR2-2 intra-frequency NR case	4792

A.15.1.1.1.1	Test Purpose and Environment	4792

A.15.1.1.1.2	Test Parameters	4792

A.15.1.1.1.3	Test Requirements	4796

A.15.1.2	Cell reselection to FR2-2 inter-frequency NR case	4796

A.15.1.2.1	Test Purpose and Environment	4796

A.15.1.2.2	Test Parameters	4796

A.15.1.2.3	Test Requirements	4800

A.15.1.3	Cell reselection to FR2-2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	4800

A.15.1.3.1	Test Purpose and Environment	4800

A.15.1.3.2	Test Parameters	4800

A.15.1.3.3	Test Requirements	4803

A.15.1.4	Cell reselection to FR2-2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	4803

A.15.1.4.1	Test Purpose and Environment	4803

A.15.1.4.2	Test Parameters	4803

A.15.1.4.3	Test Requirements	4806

A.15.1.5	Cell reselection to FR2-2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	4806

A.15.1.5.1	Test Purpose and Environment	4806

A.15.1.5.2	Test Parameters	4806

A.15.1.5.3	Test Requirements	4810

A.15.1.6	Cell reselection to FR2-2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	4810

A.15.1.6.1	Test Purpose and Environment	4810

A.15.1.6.2	Test Parameters	4810

A.15.1.6.3	Test Requirements	4813

A.15.2	Signaling characteristics	4814

A.15.2.1	SCell Activation and Deactivation Delay	4814

A.15.2.1.1	SCell Activation and deactivation for SCell in FR2-2 intra-band in non-DRX	4814

A.15.2.1.1.1	Test Purpose and Environment	4814

A.15.2.1.1.2	Test Requirements	4816

A.15.2.1.2	SCell Activation and deactivation for FR1+FR2-2 inter-band with target SCell in FR2-2	4816

A.15.2.1.2.1	Test Purpose and Environment	4816

A.15.2.1.2.2	Test Requirements	4821

A.15.2.1.3	SCell Activation and deactivation for SCell in FR2-2 inter-band in non-DRX	4822

A.15.2.1.3.1	Test Purpose and Environment	4822

A.15.2.1.3.2	Test Requirements	4825

A.15.2.1.4	Direct SCell activation at SCell addition of known SCell in FR2-2	4826

A.15.2.1.4.1	Test Purpose and Environment	4826

A.15.2.1.4.2	Test Requirements	4829

A.15.2.1.5	Direct SCell activation at handover with known SCell in FR2-2	4830

A.15.2.1.5.1	Test Purpose and Environment	4830

A.15.2.1.5.2	Test Requirements	4834

A.15.3	RRC_CONNECTED state mobility	4835

A.15.3.1	Handover	4835

A.15.3.1.1	Intra-frequency handover from FR2-2 carrier with CCA to FR2-2 carrier with CCA; unknown target cell	4835

A.15.3.1.1.1	Test Purpose and Environment	4835

A.15.3.1.1.2	Test Parameters	4835

A.15.3.1.1.3	Test Requirements	4839

A.15.3.1.2	Inter-frequency handover from FR1 to FR2-2 carrier with CCA; unknown target cell	4839

A.15.3.1.2.1	Test Purpose and Environment	4839

A.15.3.1.2.2	Test Parameters	4839

A.15.3.1.2.3	Test Requirements	4843

A.15.4	Measurement procedure	4844

A.15.4.1	Intra-frequency Measurements	4844

A.15.4.1.1	SA event triggered reporting test without gap under non-DRX for FR2-2 with CCA	4844

A.15.4.1.1.1	Test purpose and Environment	4844

A.15.4.1.1.2	Test Requirements	4846

A.15.4.2	Inter-frequency Measurements	4847

A.15.4.2.1	SA event triggered reporting tests for FR2-2 with CCA without SSB time index detection when DRX is not used (PCell in FR2-2)	4847

A.15.4.2.1.1	Test Purpose and Environment	4847

A.15.4.2.1.2	Test Requirements	4850

A.16	NR standalone tests with all NR cells in FR1 for RedCap	4851

A.16.1	SA: RRC_IDLE state mobility for RedCap	4851

A.16.1.1	Cell re-selection to NR	4851

A.16.1.1.1	Cell reselection to FR1 intra-frequency NR case for 1 Rx UE	4851

A.16.1.1.1.1	Test Purpose and Environment	4851

A.16.1.1.1.2	Test Parameters	4851

A.16.1.1.1.3	Test Requirements	4855

A.16.1.1.2	Cell reselection to FR1 intra-frequency NR case for 2 Rx UE	4855

A.16.1.1.2.1	Test Purpose and Environment	4855

A.16.1.1.2.2	Test Parameters	4855

A.16.1.1.2.3	Test Requirements	4859

A.16.1.1.3	Cell reselection to FR1 inter-frequency NR case for 1 Rx UE	4859

A.16.1.1.3.1	Test Purpose and Environment	4859

A.16.1.1.3.2	Test Parameters	4859

A.16.1.1.3.3	Test Requirements	4864

A.16.1.1.4	Cell reselection to FR1 inter-frequency NR case for 2 Rx UE	4864

A.16.1.1.4.1	Test Purpose and Environment	4864

A.16.1.1.4.2	Test Parameters	4864

A.16.1.1.4.3	Test Requirements	4869

A.16.1.1.5	Cell reselection to FR1 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	4869

A.16.1.1.5.1	Test Purpose and Environment	4869

A.16.1.1.5.2	Test Parameters	4869

A.16.1.1.5.3	Test Requirements	4874

A.16.1.1.6	Cell reselection to FR1 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	4874

A.16.1.1.6.1	Test Purpose and Environment	4874

A.16.1.1.6.2	Test Parameters	4874

A.16.1.1.6.3	Test Requirements	4879

A.16.1.1.7	Cell reselection to FR1 inter-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	4879

A.16.1.1.7.1	Test Purpose and Environment	4879

A.16.1.1.7.2	Test Parameters	4879

A.16.1.1.7.3	Test Requirements	4884

A.16.1.1.8	Cell reselection to FR1 inter-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	4884

A.16.1.1.8.1	Test Purpose and Environment	4884

A.16.1.1.8.2	Test Parameters	4885

A.16.1.1.8.3	Test Requirements	4889

A.16.1.2	Inter-RAT E-UTRAN cell re-selection for RedCap	4890

A.16.1.2.1	Cell reselection to higher priority E-UTRAN for 1RX	4890

A.16.1.2.1.1	Test Purpose and Environment	4890

A.16.1.2.1.2	Test Parameters	4890

A.16.1.2.1.3	Test Requirements	4893

A.16.1.2.2	Cell reselection to higher priority E-UTRAN for 2RX	4894

A.16.1.2.2.1	Test Purpose and Environment	4894

A.16.1.2.2.2	Test Parameters	4894

A.16.1.2.2.3	Test Requirements	4897

A.16.1.2.3.1	Test Purpose and Environment	4898

A. 16.1.2.3.2	Test Parameters	4898

A.16.1.2.3.3	Test Requirements	4901

A.16.1.2.4.1	Test Purpose and Environment	4902

A.16.1.2.4.2	Test Parameters	4902

A.16.1.3.1.3	Test Requirements	4905

A.16.1.2.5	Cell reselection to lower priority E-UTRAN for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	4906

A.16.1.2.5.1	Test Purpose and Environment	4906

A.16.1.2.5.2	Test Parameters	4906

A.16.1.2.5.3	Test Requirements	4909

A.16.1.2.6	Cell reselection to lower priority E-UTRAN for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	4910

A.16.1.2.6.1	Test Purpose and Environment	4910

A.16.1.2.6.2	Test Parameters	4910

A.16.1.2.6.3	Test Requirements	4913

A.16.2	SA: RRC_INACTIVE state mobility for RedCap	4914

A.16.3	RRC_CONNECTED state mobility for RedCap	4914

A.16.3.1	Handover	4914

A.16.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell for 1 Rx UE	4914

A.16.3.1.1.1	Test Purpose and Environment	4914

A.16.3.1.1.2	Test Parameters	4914

A.16.3.1.1.3	Test Requirements	4918

A.16.3.1.2	Intra-frequency handover from FR1 to FR1; known target cell for 2 Rx UE	4918

A.16.3.1.2.1	Test Purpose and Environment	4918

A.16.3.1.2.2	Test Parameters	4918

A.16.3.1.2.3	Test Requirements	4922

A.16.3.1.3	Intra-frequency handover from FR1 to FR1; unknown target cell for 1 Rx UE	4922

A.16.3.1.3.1	Test Purpose and Environment	4922

A.16.3.1.3.2	Test Parameters	4922

A.16.3.1.3.3	Test Requirements	4926

A.16.3.1.4	Intra-frequency handover from FR1 to FR1; unknown target cell for 2 Rx UE	4926

A.16.3.1.4.1	Test Purpose and Environment	4926

A.16.3.1.4.2	Test Parameters	4927

A.16.3.1.5	Inter-frequency handover from FR1 to FR1; unknown target cell for 1 Rx UE	4930

A.16.3.1.5.1	Test Purpose and Environment	4930

A.16.3.1.5.2	Test Parameters	4930

A.16.3.1.5.3	Test Requirements	4934

A.16.3.1.6	Inter-frequency handover from FR1 to FR1; unknown target cell for 2 Rx UE	4934

A.16.3.1.6.1	Test Purpose and Environment	4934

A.16.3.1.6.2	Test Parameters	4934

A.16.3.1.6.3	Test Requirements	4938

A.16.3.1.7	SA NR - E-UTRAN handover for 1Rx UE	4938

A.16.3.1.7.1	Test Purpose and Environment	4938

A.16.3.1.7.2	Test Requirements	4944

A.16.3.1.8	 SA NR - E-UTRAN handover for 2Rx UE	4944

A.16.3.1.8.1	Test Purpose and Environment	4944

A.16.3.1.8.2	Test Requirements	4950

A.16.3.1.9	SA NR - E-UTRAN handover with unknown target cell for 1 Rx UE	4950

A.16.3.1.9.1	Test Purpose and Environment	4950

A.16.3.1.9.2	Test Requirements	4957

A.16.3.1.10	SA NR - E-UTRAN handover with unknown target cell for 2 Rx UE	4957

A.16.3.1.10.1	Test Purpose and Environment	4957

A.16.3.1.10.2	Test Requirements	4963

A.16.3.2	RRC Connection Mobility Control	4963

A.16.3.2.1	SA: RRC Re-establishment	4963

A.16.3.2.1.1	Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE	4963

A.16.3.2.1.2	Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE	4967

A.16.3.2.1.3	Inter-frequency RRC Re-establishment in FR1 for 1 Rx UE	4971

A.16.3.2.1.4	Inter-frequency RRC Re-establishment in FR1 for 2 Rx UE	4976

A.16.3.2.1.5	Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE without serving cell timing	4981

A.16.3.2.1.6	Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE without serving cell timing	4985

A.16.3.2.2	Random Access	4989

A.16.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	4989

A.16.3.2.2.2	4-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	4995

A.16.3.2.2.3	4-step RA type non-contention based random access test in FR1 for NR standalone for 1 Rx UE	5000

A.16.3.2.2.5	2-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	5010

A.16.3.2.2.6	2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	5014

A.16.3.2.2.7	2-step RA type non-contention based test in FR1 for NR standalone for 1 RX UE	5018

A.16.3.2.2.8	2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE	5022

A.16.3.2.3	SA: RRC Connection Release with Redirection	5026

A.16.3.2.3.1	Redirection from NR in FR1 to NR in FR1 for 1 Rx UE	5026

A.16.3.2.3.2	Redirection from NR in FR1 to NR in FR1 for 2 Rx UE	5030

A.16.3.2.3.3	Redirection from NR in FR1 to E-UTRAN for 1 Rx UE	5034

A.16.3.2.3.4	Redirection from NR in FR1 to E-UTRAN for 2 Rx UE	5041

A.16.4	Timing for RedCap	5048

A.16.4.1	UE transmit timing	5048

A.16.4.1.1	NR UE Transmit Timing Test for FR1 for 1Rx RedCap UE	5048

A.16.4.1.1.1	Test Purpose and environment	5048

A.16.4.1.1.2	Test requirements	5050

A.16.4.1.2	NR UE Transmit Timing Test for FR1 for 2Rx RedCap UE	5051

A.16.4.1.2.1	Test Purpose and environment	5051

A.16.4.1.2.2	Test requirements	5053

A.16.4.2	UE timer accuracy	5053

A.16.4.3	Timing advance	5053

A.16.4.3.1	SA FR1 timing advance adjustment accuracy for 1 Rx UE	5053

A.16.4.3.1.1	Test Purpose and Environment	5053

A.16.4.3.1.2	Test Parameters	5053

A.16.4.3.1.3	Test Requirements	5057

A.16.4.3.2	SA FR1 timing advance adjustment accuracy for 2 Rx UE	5057

A.16.4.3.2.1	Test Purpose and Environment	5057

A.16.4.3.2.2	Test Parameters	5058

A.16.4.3.2.3	Test Requirements	5061

A.16.5	Signalling characteristics for RedCap	5062

A.16.5.1	Radio link Monitoring	5062

A.16.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	5062

A.16.5.1.1.1	Test Purpose and Environment	5062

A.16.5.1.1.2	Test Requirements	5067

A.16.5.1.2	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	5067

A.16.5.1.2.1	Test Purpose and Environment	5067

A.16.5.1.2.2	Test Requirements	5073

A.16.5.1.3	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	5073

A.16.5.1.3.1	Test Purpose and Environment	5073

A.16.5.1.3.2	Test Requirements	5079

A.16.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	5079

A.16.5.1.4.1	Test Purpose and Environment	5079

A.16.5.1.4.2	Test Requirements	5085

A.16.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	5085

A.16.5.1.5.1	Test Purpose and Environment	5085

A.16.5.1.5.2	Test Requirements	5091

A.16.5.1.6	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	5091

A.16.5.1.6.1	Test Purpose and Environment	5091

A.16.5.1.6.2	Test Requirements	5096

A.16.5.1.7	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	5096

A.16.5.1.7.1	Test Purpose and Environment	5096

A.16.5.1.7.2	Test Requirements	5101

A.16.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	5101

A.16.5.1.8.1	Test Purpose and Environment	5101

A.16.5.1.8.2	Test Requirements	5107

A.16.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	5107

A.16.5.1.9.1	Test Purpose and Environment	5107

A.16.5.1.9.2	Test Requirements	5113

A.16.5.1.10	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	5113

A.16.5.1.10.1	Test Purpose and Environment	5113

A.16.5.1.10.2	Test Requirements	5119

A.16.5.1.11	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	5119

A.16.5.1.11.1	Test Purpose and Environment	5119

A.16.5.1.11.2	Test Requirements	5125

A.16.5.1.12	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	5125

A.16.5.1.12.1	Test Purpose and Environment	5125

A.16.5.1.12.2	Test Requirements	5130

A.16.5.1.13	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	5130

A.16.5.1.13.1	Test Purpose and Environment	5130

A.16.5.1.13.2	Test Requirements	5136

A.16.5.1.14	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	5136

A.16.5.1.14.1	Test Purpose and Environment	5136

A.16.5.1.14.2	Test Requirements	5142

A.16.5.1.15	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	5142

A.16.5.1.15.1	Test Purpose and Environment	5142

A.16.5.1.15.2	Test Requirements	5148

A.16.5.1.16	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	5148

A.16.5.1.16.1	Test Purpose and Environment	5148

A.16.5.1.16.2	Test Requirements	5154

A.16.5.2	Beam Failure Detection and Link recovery procedures	5154

A.16.5.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 1 Rx UE	5154

A.16.5.2.1.1	Test Purpose and Environment	5154

A.16.5.2.1.2	Test Requirements	5160

A.16.5.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	5160

A.16.5.2.2.1	Test Purpose and Environment	5160

A.16.5.2.2.2	Test Requirements	5166

A.16.5.2.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 1 Rx UE	5166

A.16.5.2.3.1	Test Purpose and Environment	5166

A.16.5.2.3.2	Test Requirements	5173

A.16.5.2.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 2 Rx UE	5173

A.16.5.2.4.1	Test Purpose and Environment	5173

A.16.5.2.4.2	Test Requirements	5179

A.16.5.2.5	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 1 Rx UE	5179

A.16.5.2.5.1	Test Purpose and Environment	5179

A.16.5.2.5.2	Test Requirements	5185

A.16.5.2.6	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE	5185

A.16.5.2.6.1	Test Purpose and Environment	5185

A.16.5.2.6.2	Test Requirements	5191

A.16.5.2.7	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 1 Rx UE	5191

A.16.5.2.7.1	Test Purpose and Environment	5191

A.16.5.2.7.2	Test Requirements	5197

A.16.5.2.8	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE	5197

A.16.5.2.8.1	Test Purpose and Environment	5197

A.16.5.2.8.2	Test Requirements	5203

A.16.5.3	Active BWP switch	5203

A.16.5.3.1	DCI-based and Timer-based Active BWP Switch	5203

A.16.5.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA for 1 Rx UE	5203

A.16.5.3.1.1.1	Test Purpose and Environment	5203

A.16.5.3.1.1.2	Test Requirements	5208

A.16.5.3.1.2	NR FR1 DL active BWP switch with non-DRX in SA for 2 Rx UE	5208

A.16.5.3.1.1.2	Test Purpose and Environment	5208

A.16.5.3.1.2.2	Test Requirements	5213

A.16.5.3.2	RRC-based Active BWP Switch	5213

A.16.5.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA for 1 Rx UE	5213

A.16.5.3.2.1.1	Test Purpose and Environment	5213

A.16.5.3.2.1.2	Test Requirements	5217

A.16.5.3.2.2	NR FR1 DL active BWP switch of Cell with non-DRX in SA for 2 Rx UE	5217

A.16.5.3.2.2.1	Test Purpose and Environment	5217

A.16.5.3.2.2.2	Test Requirements	5221

A.16.5.8	UE specific CBW change	5221

A.16.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX for 1 Rx UE	5221

A.16.5.8.1.1	Test Purpose and Environment	5221

A.16.5.8.1.2	Test Requirements	5225

A.16.5.8.2	UE specific CBW change on PCell in FR1 in non-DRX for 2 Rx UE	5225

A.16.5.8.2.1	Test Purpose and Environment	5225

A.6.5.8.1.2	Test Requirements	5229

A.16.6	Measurement procedure for RedCap	5229

A.16.6.1	Intra-frequency Measurements	5229

A.16.6.1.1	SA event triggered reporting tests without gap under non-DRX for 1 Rx UE	5229

A.16.6.1.1.1	Test purpose and Environment	5229

A.16.6.1.1.2	Test parameters	5229

A.16.6.1.1.3	Test Requirements	5233

A.16.6.1.2	SA event triggered reporting tests without gap under non-DRX for 2 Rx UE	5233

A.16.6.1.2.1	Test purpose and Environment	5233

A.16.6.1.2.2	Test parameters	5233

A.16.6.1.2.3	Test Requirements	5237

A.16.6.1.3	SA event triggered reporting tests without gap under DRX for 1 Rx UE	5237

A.16.6.1.3.1	Test purpose and Environment	5237

A.16.6.1.3.2	Test parameters	5237

A.16.6.1.3.3	Test Requirements	5241

A.16.6.1.4	SA event triggered reporting tests without gap under DRX for 2 Rx UE	5241

A.16.6.1.4.1	Test purpose and Environment	5241

A.16.6.1.4.2	Test parameters	5241

A.16.6.1.4.3	Test Requirements	5245

A.16.6.1.5	SA event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE	5245

A.16.6.1.5.1	Test purpose and Environment	5245

A.16.6.1.5.2	Test parameters	5245

A.16.6.1.5.3	Test Requirements	5249

A.16.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX for 2 Rx UE	5249

A.16.6.1.6.1	Test purpose and Environment	5249

A.16.6.1.6.2	Test parameters	5249

A.16.6.1.6.3	Test Requirements	5253

A.16.6.1.7	SA event triggered reporting tests with per-UE gaps under DRX for 1 Rx UE	5253

A.16.6.1.7.1	Test purpose and Environment	5253

A.16.6.1.7.2	Test parameters	5253

A.16.6.1.7.3	Test Requirements	5257

A.16.6.1.8	SA event triggered reporting tests with per-UE gaps under DRX for 2 Rx UE	5257

A.16.6.1.8.1	Test purpose and Environment	5257

A.16.6.1.8.2	Test parameters	5257

A.16.6.1.8.3	Test Requirements	5261

A.16.6.1.9	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 1 Rx UE	5261

A.16.6.1.9.1	Test purpose and Environment	5261

A.16.6.1.9.2	Test parameters	5261

A.16.6.1.9.3	Test Requirements	5264

A.16.6.1.10	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 2 Rx UE	5264

A.16.6.1.10.1	Test purpose and Environment	5264

A.16.6.1.10.2	Test parameters	5264

A.16.6.1.10.3	Test Requirements	5266

A.16.6.1.11	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 1 Rx UE	5267

A.16.6.1.11.1	Test purpose and Environment	5267

A.16.6.1.11.2	Test parameters	5267

A.16.6.1.11.3	Test Requirements	5269

A.16.6.1.12	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 2 Rx UE	5270

A.16.6.1.12.1	Test purpose and Environment	5270

A.16.6.1.12.2	Test parameters	5270

A.16.6.1.12.3	Test Requirements	5272

A.16.6.2	Inter-frequency Measurements	5273

A.16.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE	5273

A.16.6.2.1.1	Test Purpose and Environment	5273

A.16.6.2.1.2	Test Requirements	5278

A.16.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE	5278

A.16.6.2.2.1	Test Purpose and Environment	5278

A.16.6.2.2.2	Test Requirements	5283

A.16.6.2.3	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE	5283

A.16.6.2.3.1	Test Purpose and Environment	5283

A.16.6.2.3.2	Test Requirements	5286

A.16.6.2.4	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE	5287

A.16.6.2.4.1	Test Purpose and Environment	5287

A.16.6.2.4.2	Test Requirements	5290

A.16.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 1 Rx UE	5291

A.16.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 2 Rx UE	5291

A.16.6.2.7	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 1 Rx UE	5291

A.16.6.2.7.1	Test Purpose and Environment	5291

A.16.6.2.7.2	Test Requirements	5295

A.16.6.2.8	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 2 Rx UE	5296

A.16.6.2.8.1	Test Purpose and Environment	5296

A.16.6.2.8.2	Test Requirements	5300

A.16.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern for 1 Rx UE	5301

A.16.6.2.9.1	Test Purpose and Environment	5301

A.16.6.2.9.2	Test Requirements	5303

A.16.6.2.10	SA event triggered reporting tests with additional mandatory gap pattern for 2 Rx UE	5304

A.16.6.2.10.1	Test Purpose and Environment	5304

A.16.6.2.10.2	Test Requirements	5306

A.16.6.2.11	SA event triggered reporting tests for FR1 when DRX is used for 1 Rx UE	5307

A.16.6.2.11.1	Test Purpose and Environment	5307

A.16.6.2.11.2	Test Requirements	5310

A.16.6.2.12	SA event triggered reporting tests for FR1 when DRX is used for 2 Rx UE	5310

A.16.6.2.12.1	Test Purpose and Environment	5310

A.16.6.2.12.2	Test Requirements	5314

A.16.6.3	Inter-RAT Measurements	5315

A.16.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for 1 Rx UE	5315

A.16.6.3.1.1	Test purpose and Environment	5315

A.16.6.3.1.2	Test Requirements	5320

A.16.6.3.2	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for 2 Rx UE	5320

A.16.6.3.2.1	Test purpose and Environment	5320

A.16.6.3.2.2	Test Requirements	5325

A.16.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for 1 Rx UE	5325

A.16.6.3.3.1	Test purpose and Environment	5325

A.16.6.3.3.2	Test Requirements	5330

A.16.6.3.4	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for 2 Rx UE	5330

A.16.6.3.4.1	Test purpose and Environment	5330

A.16.6.3.4.2	Test Requirements	5335

A.16.6.4	L1-RSRP measurement for beam reporting	5335

A.16.6.4.1	SSB based L1-RSRP measurement when DRX is not used for 1 Rx UE	5335

A.16.6.4.1.1	Test Purpose and Environment	5335

A.16.6.4.1.2	Test parameters	5335

A.16.6.4.1.3	Test Requirements	5338

A.16.6.4.2	SSB based L1-RSRP measurement when DRX is not used for 2 Rx UE	5338

A.16.6.4.2.1	Test Purpose and Environment	5338

A.16.6.4.2.2	Test parameters	5339

A.16.6.4.2.3	Test Requirements	5342

A.16.6.4.3	SSB based L1-RSRP measurement when DRX is used for 1 Rx UE	5342

A.16.6.4.3.1	Test Purpose and Environment	5342

A.16.6.4.3.2	Test parameters	5343

A.16.6.4.3.3	Test Requirements	5346

A.16.6.4.4	SSB based L1-RSRP measurement when DRX is used for 2 Rx UE	5346

A.16.6.4.4.1	Test Purpose and Environment	5346

A.16.6.4.4.2	Test parameters	5347

A.16.6.4.4.3	Test Requirements	5350

A.16.6.4.5	CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE	5350

A.16.6.4.5.1	Test Purpose and Environment	5350

A.16.6.4.5.2	Test parameters	5351

A.16.6.4.5.3	Test Requirements	5354

A.16.6.4.6	CSI-RS based L1-RSRP measurement when DRX is not used for 2 Rx UE	5354

A.16.6.4.6.1	Test Purpose and Environment	5354

A.16.6.4.6.2	Test parameters	5355

A.16.6.4.6.3	Test Requirements	5358

A.16.6.4.7	CSI-RS based L1-RSRP measurement when DRX is used for 1 Rx UE	5358

A.16.6.4.7.1	Test Purpose and Environment	5358

A.16.6.4.7.2	Test parameters	5359

A.16.6.4.7.3	Test Requirements	5362

A.16.6.4.8	CSI-RS based L1-RSRP measurement when DRX is used for 2 Rx UE	5362

A.16.6.4.8.1	Test Purpose and Environment	5362

A.16.6.4.8.2	Test parameters	5363

A.16.6.4.8.3	Test Requirements	5366

A.16.6.7	NR measurements with autonomous gaps	5366

A.16.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1 for 1 Rx UE	5366

A.16.6.7.1.1	Test Purpose and Environment	5366

A.16.6.7.1.2	Test Parameters	5366

A.16.6.7.1.3	Test Requirements	5369

A.16.6.7.2	SA intra-frequency CGI identification of NR neighbor cell in FR1 for 2 Rx UE	5369

A.16.6.7.2.1	Test Purpose and Environment	5369

A.16.6.7.2.2	Test Parameters	5369

A.16.6.7.2.3	Test Requirements	5372

A.16.6.7.3	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 1 Rx UE	5372

A.16.6.7.3.1	Test Purpose and Environment	5372

A.16.6.7.3.2	Test Requirements	5379

A.16.6.7.4	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 2 Rx UE	5379

A.16.6.7.4.1	Test Purpose and Environment	5379

A.16.6.7.4.2	Test Requirements	5386

A.16.7	Measurement Performance requirements for RedCap	5386

A.16.7.1	SS-RSRP	5386

A.16.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5386

A.16.7.1.1.1	Test Purpose and Environment	5386

A.16.7.1.1.2	Test parameters	5386

A.16.7.1.1.3	Test Requirements	5391

A.16.7.1.2	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 2Rx UE	5391

A.16.7.1.2.1	Test Purpose and Environment	5391

A.16.7.1.2.2	Test parameters	5391

A.16.7.1.2.3	Test Requirements	5396

A.16.7.1.3	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5396

A.16.7.1.3.1	Test Purpose and Environment	5396

A.16.7.1.3.2	Test parameters	5396

A.16.7.1.3.3	Test Requirements	5400

A.16.7.1.4	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5400

A.16.7.1.4.1	Test Purpose and Environment	5400

A.16.7.1.4.2	Test parameters	5401

A.16.7.1.4.3	Test Requirements	5405

A.16.7.2	SS-RSRQ	5405

A.16.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5405

A.16.7.2.1.1	Test Purpose and Environment	5405

A.16.7.2.1.2	Test Parameters	5406

A.16.7.2.1.3	Test Requirements	5411

A.16.7.2.2	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5411

A.16.7.2.2.1	Test Purpose and Environment	5411

A.16.7.2.2.2	Test Parameters	5411

A.16.7.2.2.3	Test Requirements	5416

A.16.7.2.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5416

A.16.7.2.3.1	Test Purpose and Environment	5416

A.16.7.2.3.2	Test parameters	5416

A.16.7.2.3.3	Test Requirements	5421

A.16.7.2.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5421

A.16.7.2.4.1	Test Purpose and Environment	5421

A.16.7.2.4.2	Test parameters	5421

A.16.7.2.4.3	Test Requirements	5427

A.16.7.3	SS-SINR	5427

A.16.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5427

A.16.7.3.1.1	Test Purpose and Environment	5427

A.16.7.3.1.2	Test parameters	5428

A.16.7.3.1.3	Test Requirements	5432

A.16.7.3.2	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5433

A.16.7.3.2.1	Test Purpose and Environment	5433

A.16.7.3.2.2	Test parameters	5433

A.16.7.3.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5437

A.16.7.3.3.1	Test Purpose and Environment	5437

A.16.7.3.3.2	Test parameters	5437

A.16.7.3.3.3	Test Requirements	5442

A.16.7.3.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5443

A.16.7.3.4.1	Test Purpose and Environment	5443

A.16.7.3.4.2	Test parameters	5443

A.16.7.3.4.3	Test Requirements	5447

A.16.7.4	L1-RSRP measurement for beam reporting	5448

A.16.7.4.1	SSB based L1-RSRP measurement for 1 Rx UE	5448

A.16.7.4.2	SSB based L1-RSRP measurement for 2 Rx UE	5448

A.16.7.4.3	CSI-RS based L1-RSRP measurement on resource set with repetition off for 1 Rx UE	5448

A.16.7.4.4	CSI-RS based L1-RSRP measurement on resource set with repetition off for 2 Rx UE	5448

A.16.7.5	E-UTRAN RSRP	5448

A.16.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5448

A.16.7.5.1.1	Test Purpose and Environment	5448

A.16.7.5.1.2	Test parameters	5448

A.16.7.5.1.3	Test Requirements	5454

A.16.7.5.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5455

A.16.7.5.2.1	Test Purpose and Environment	5455

A.16.7.5.2.2	Test parameters	5455

A.16.7.5.2.3	Test Requirements	5461

A.16.7.6	E-UTRAN RSRQ	5462

A.16.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5462

A.16.7.6.1.1	Test Purpose and Environment	5462

A.16.7.6.1.2	Test parameters	5462

A.16.7.6.1.3	Test Requirements	5468

A.16.7.6.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5469

A.16.7.6.2.1	Test Purpose and Environment	5469

A.16.7.6.2.2	Test parameters	5469

A.16.7.6.2.3	Test Requirements	5475

A.16.7.7	E-UTRAN RS-SINR	5476

A.16.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5476

A.16.7.7.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5476

A.17	NR standalone tests with one or more NR cells in FR2 for RedCap	5476

A.17.1	SA: RRC_IDLE state mobility for RedCap	5476

A.17.1.1	Cell re-selection to NR	5476

A.17.1.1.1	Cell reselection to FR2 intra-frequency NR case for 2 Rx	5476

A.17.1.1.1.1	Test Purpose and Environment	5476

A.17.1.1.1.2	Test Parameters	5476

A.17.1.1.1.3	Test Requirements	5480

A.17.1.1.2	Cell reselection to FR2 inter-frequency NR case	5480

A.17.1.1.2.1	Test Purpose and Environment	5480

A.17.1.1.2.2	Test Parameters	5480

A.17.1.1.2.3	Test Requirements	5484

A.17.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5484

A.17.1.1.3.1	Test Purpose and Environment	5484

A.17.1.1.3.2	Test Parameters	5484

A.17.1.1.3.3	Test Requirements	5487

A.17.1.1.4	Cell reselection to FR2 inter-frequency NR case for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE	5487

A.17.1.1.4.1	Test Purpose and Environment	5487

A.17.1.1.4.2	Test Parameters	5487

A.17.1.1.4.3	Test Requirements	5490

A.17.2	SA: RRC_INACTIVE state mobility for RedCap	5490

A.17.3	RRC_CONNECTED state mobility for RedCap	5490

A.17.3.1	Handover for RedCap	5490

A.17.3.1.1	Intra-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	5490

A.17.3.1.1.1	Test Purpose and Environment	5490

A.17.3.1.1.2	Test Parameters	5491

A.17.3.1.1.3	Test Requirements	5493

A.17.3.1.2	Inter-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	5493

A.17.3.1.2.1	Test Purpose and Environment	5493

A.17.3.1.2.2	Test Parameters	5493

A.17.3.1.2.3 Test Requirements	5495

A.17.3.2	RRC Connection Mobility Control for RedCap	5496

A.17.3.2.1	SA: RRC Re-establishment	5496

A.17.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	5496

A.17.3.2.1.1.1	Test Purpose and Environment	5496

A.17.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	5499

A.17.3.2.1.2.1	Test Purpose and Environment	5499

A.17.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	5502

A.17.3.2.1.3.1	Test Purpose and Environment	5502

A.17.3.2.1.3.2	Test Requirements	5504

A.17.3.2.2	Random Access	5505

A.17.3.2.2.1	4-step RA type contention based random access test in FR2 for NR Standalone	5505

A.17.3.2.2.1.1	Test Purpose and Environment	5505

A.17.3.2.2.1.2	Test Requirements	5507

A.17.3.2.2.2	4-step RA type non-contention based random access test in FR2 for NR Standalone	5509

A.17.3.2.2.2.1	Test Purpose and Environment	5509

A.17.3.2.2.2.2	Test Requirements	5511

A.17.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	5513

A.17.3.2.2.3.1	Test Purpose and Environment	5513

A.17.3.2.2.3.2	Test Requirements	5515

A.17.3.2.2.4	2-step RA type non-contention based random access test in FR2 for NR Standalone	5516

A.17.3.2.2.4.1	Test Purpose and Environment	5516

A.17.3.2.2.4.2	Test Requirements	5518

A.17.3.2.3	SA: RRC Connection Release with Redirection	5519

A.17.3.2.3.1	Redirection from NR in FR2 to NR in FR2	5519

A.17.3.2.3.1.1	Test Purpose and Environment	5519

A.17.3.2.3.1.2	Test Parameters	5519

A.17.3.2.3.1.3	Test Requirements	5523

A.17.4	Timing	5523

A.17.4.1	UE transmit timing	5523

A.17.4.1.1	NR UE Transmit Timing Test for FR2	5523

A.17.4.1.1.1	Test Purpose and environment	5523

A.17.4.1.1.2	Test requirements	5526

A.17.4.2	UE timer accuracy	5527

A.17.4.3	Timing advance	5527

A.17.4.3.1	SA FR2 timing advance adjustment accuracy	5527

A.17.4.3.1.1	Test Purpose and Environment	5527

A.17.4.3.1.2	Test Parameters	5527

A.17.4.3.1.3 Test Requirements	5530

A.17.4.2	UE timer accuracy	5530

A.17.4.3	Timing advance	5530

A.17.4.3.1	SA FR2 timing advance adjustment accuracy	5530

A.17.5	Signaling characteristics for RedCap	5530

A.17.5.1	Radio link Monitoring for RedCap	5530

A.17.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	5530

A.17.5.1.1.1	Test Purpose and Environment	5530

A.17.5.1.1.2	Test Requirements	5533

A.17.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	5534

A.17.5.1.2.1	Test Purpose and Environment	5534

A.17.5.1.2.2	Test Requirements	5538

A.17.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	5539

A.17.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	5539

A.17.5.1.4.1	Test Purpose and Environment	5539

A.17.5.1.4.2	Test Requirements	5543

A.17.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	5543

A.17.5.1.5.1	Test Purpose and Environment	5543

A.17.5.1.5.2	Test Requirements	5548

A.17.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	5548

A.17.5.1.6.1	Test Purpose and Environment	5548

A.17.5.1.6.2	Test Requirements	5552

A.17.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	5552

A.17.5.1.7.1	Test Purpose and Environment	5552

A.17.5.1.7.2	Test Requirements	5556

A.17.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	5556

A.17.5.1.8.1	Test Purpose and Environment	5556

A.17.5.1.8.2	Test Requirements	5561

A.17.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	5561

A.17.5.1.9.1	Test Purpose and Environment	5561

A.17.5.1.9.2	Test Requirements	5564

A.17.5.2	Beam Failure Detection and Link recovery procedures	5564

A.17.5.2.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	5564

A.17.5.2.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	5564

A.17.5.2.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	5564

A.17.5.2.3.1	Test Purpose and Environment	5564

A.17.5.2.3.2	Test Requirements	5568

A.17.5.2.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	5568

A.17.5.2.4.1	Test Purpose and Environment	5568

A.17.5.2.4.2	Test Requirements	5572

A.17.5.2.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	5572

A.17.5.2.5.1	Test Purpose and Environment	5572

A.17.5.2.5.2	Test Requirements	5576

A.17.5.3	Active BWP switch for RedCap	5576

A.17.5.3.1	DCI-based and Timer-based Active BWP Switch	5576

A.17.5.3.1.1	NR FR2 DL active BWP switch with non-DRX in SA	5576

A.17.5.3.1.1.1	Test Purpose and Environment	5576

A.17.5.3.1.1.2	Test Requirements	5579

A.17.5.3.2	RRC-based Active BWP Switch	5579

A.17.5.3.2.1	NR FR2 DL active BWP switch of PCell with non-DRX in SA	5579

A.17.5.3.2.1.1	Test Purpose and Environment	5579

A.17.5.3.2.1.2	Test Requirements	5582

A.17.5.4	Active TCI state switch delay	5583

A.17.5.4.1	MAC-CE based active TCI state switch	5583

A.17.5.4.1.1	NR PCell FR2 active TCI state switch for a known TCI state	5583

A.17.5.4.1.1.1	Test Purpose and Environment	5583

A.17.5.4.1.1.2	Test Requirements	5586

A.17.5.4.2	RRC based active TCI state switch	5586

A.17.5.4.2.1	NR PCell FR2 active TCI state switch for a known TCI state	5586

A.17.5.4.2.1.1	Test Purpose and Environment	5586

A.17.5.4.2.1.2	Test Requirements	5590

A.17.5.5	Uplink spatial relation switch delay	5590

A.17.5.5.1	MAC-CE based Spatial Relation switch	5590

A.17.5.5.1.1	 NR PCell FR2 spatial relation associated with known DL-RS	5590

A.17.5.5.1.1.1	Test Purpose and Environment	5590

A.17.5.5.1.1.2	Test Requirements	5593

A.17.5.5.2	RRC based spatial relation switch	5593

A.17.5.5.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	5593

A.17.5.5.2.1.2	Test Requirements	5596

A.17.5.6	UE specific CBW change	5596

A.17.5.6.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	5596

A.17.5.6.1.1	Test Purpose and Environment	5596

A.17.5.6.1.2	Test Requirements	5599

A.17.6	Measurement procedure for RedCap	5600

A.17.6.1	Intra-frequency Measurements	5600

A.17.6.1.1	SA event triggered reporting test without gap under non-DRX	5600

A.17.6.1.1.1	Test purpose and Environment	5600

A.17.6.1.1.2	Test Requirements	5603

A.17.6.1.2	SA event triggered reporting test without gap under DRX	5603

A.17.6.1.2.1	Test purpose and Environment	5603

A.7.6.1.2.2	Test Requirements	5603

A.17.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	5604

A.17.6.1.3.1	Test purpose and Environment	5604

A.17.6.1.3.2	Test Requirements	5608

A.17.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	5608

A.17.6.1.4.1	Test purpose and Environment	5608

A.17.6.1.4.2	Test Requirements	5611

A.17.6.2	Inter-frequency Measurements	5612

A.17.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)	5612

A.17.6.2.1.1	Test Purpose and Environment	5612

A.17.6.2.1.2	Test Requirements	5615

A.17.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)	5615

A.17.6.2.2.1	Test Purpose and Environment	5615

A.17.6.2.2.2	Test Requirements	5619

A.17.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)	5619

A.17.6.2.3.1	Test Purpose and Environment	5619

A.17.6.2.3.2	Test Requirements	5623

A.17.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2) for 2 RX UE	5623

A.17.6.2.4.1	Test Purpose and Environment	5623

A.17.6.2.4.2	Test Requirements	5627

A.17.6.3	L1-RSRP measurement for beam reporting	5627

A.17.6.3.1	SSB based L1-RSRP measurement when DRX is not used	5627

A.17.6.3.1.1	Test Purpose and Environment	5627

A.17.6.3.1.2	Test parameters	5628

A.17.6.3.1.3	Test Requirements	5628

A.17.6.3.2	SSB based L1-RSRP measurement when DRX is used	5628

A.17.6.3.2.1	Test Purpose and Environment	5628

A.17.6.3.2.2	Test parameters	5628

A.17.6.3.2.3	Test Requirements	5630

A.17.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	5630

A.17.6.3.3.1	Test Purpose and Environment	5630

A.17.6.3.3.2	Test parameters	5631

A.17.6.3.3.3	Test Requirements	5633

A.17.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	5634

A.17.6.3.4.1	Test Purpose and Environment	5634

A.17.6.3.4.2	Test parameters	5634

A.7.6.3.3.3	Test Requirements	5636

A.17.6.4.1	SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2) for 2 RX UE	5637

A.17.6.4.1.1	Test Purpose and Environment	5637

A.17.6.4.1.2	Test Requirements	5640

A.17.7	Measurement Performance requirements	5640

A.17.7.1	SS-RSRP	5640

A.17.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	5640

A.17.7.1.1.1	Test Purpose and Environment	5640

A.17.7.1.1.2	Test parameters	5640

A.17.7.1.1.3	Test Requirements	5644

A.17.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	5645

A.17.7.1.2.1	Test Purpose and Environment	5645

A.17.7.1.2.2	Test parameters	5645

A.17.7.1.2.3	Test Requirements	5649

A.17.7.2	SS-RSRQ	5650

A.17.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	5650

A.17.7.2.1.1	Test Purpose and Environment	5650

A.17.7.2.1.2	Test Parameters	5650

A.17.7.2.1.3	Test Requirements	5652

A.17.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell for 2 Rx UE	5652

A.17.7.2.2.1	Test Purpose and Environment	5652

A.17.7.2.2.2	Test parameters	5652

A.17.7.2.2.3	Test Requirements	5654

A.17.7.2.3	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	5654

A.17.7.3	L1-RSRP measurement for beam reporting	5654

A.17.7.3.1	SSB based L1-RSRP measurement	5654

A.17.7.3.1.1	Test Purpose and Environment	5654

A.17.7.3.1.2	Test parameters	5655

A.17.7.3.1.3	Test Requirements	5655

A.17.7.3.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	5655

A.17.7.3.2.1	Test Purpose and Environment	5655

A.17.7.3.2.2	Test parameters	5655

A.17.7.3.2.3	Test Requirements	5656

A.17.7.4	SS-SINR	5656

A.17.7.4	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell for 2Rx UE	5656

A.17.7.4.1.1	Test Purpose and Environment	5656

A.17.7.4.1.2	Test parameters	5656

A.17.7.4.1.3	Test Requirements	5658

A.18	E-UTRA standalone tests for NR RRM for RedCap	5658

A.18.1	RRC_IDLE state mobility	5658

A.18.1.1	Inter-RAT NR Cell re-selection	5658

A.18.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	5658

A.18.1.1.1.1	Test Purpose and Environment	5658

A.18.2.1.1.2	Test Requirements	5663

A.18.2	RRC_CONNECTED state mobility	5663

A.18.2.1	Handover	5663

A.18.2.1.1	E-UTRAN - NR handover in FR1	5663

A.18.2.1.1.1	Test Purpose and Environment	5663

A.18.2.1.1.2	Test Requirements	5668

A.18.2.2	RRC connection release with redirection	5668

A.18.2.2.1	Redirection from E-UTRA to NR FR1 for redcap UE	5668

A.18.2.2.1.1	Test Purpose and Environment	5668

A.18.2.2.1.2	Test Parameters	5668

A.18.2.2.1.3	Test Requirements	5675

A.18.3	Measurement procedure	5675

A.18.3.1	E-UTRA – NR Inter-RAT Measurements	5675

A.18.3.1.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	5675

A.18.3.1.1.1	Test Purpose and Environment	5675

A.18.3.1.1.2	Test Requirements	5681

A.18.3.1.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	5681

A.18.3.1.2.1	Test Purpose and Environment	5681

A.18.3.1.2.2	Test Requirements	5687

A.18.3.1.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	5687

A.18.3.1.3.1	Test Purpose and Environment	5687

A.18.3.1.3.2	Test Requirements	5693

A.18.3.1.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	5693

A.18.3.1.4.1	Test Purpose and Environment	5693

A.18.3.1.4.2	Test Requirements	5699

A.18.3.1.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	5699

A.18.3.1.5.1	Test Purpose and Environment	5699

A.18.3.1.5.2	Test Requirements	5701

A.18.3.1.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	5702

A.18.3.1.6.1	Test Purpose and Environment	5702

A.18.3.1.6.2	Test Requirements	5704

A.18.3.1.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	5705

A.18.3.1.7.1	Test Purpose and Environment	5705

A.18.3.1.7.2	Test Requirements	5707

A.18.3.1.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	5708

A.18.3.1.8.1	Test Purpose and Environment	5708

A.18.3.1.8.2	Test Requirements	5710

Annex B (normative): Conditions for RRM requirements applicability for operating bands	5149

B.1	Conditions for NR RRC_IDLE state mobility	5149

B.1.1	Introduction	5149

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	5149

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	5150

B.1.4	Conditions for measurements on NR intra-frequency cells for cell re-selection for RedCap	5150

B.1.5	Conditions for measurements on NR inter-frequency cells for cell re-selection for RedCap	5152

B.1.6	Conditions for measurements on NR intra-frequency cells for cell re-selection for satellite access	5152

B.1.7	Conditions for measurements on NR inter-frequency cells for cell re-selection for satellite access	5153

B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state	5153

B.2.1	Introduction	5153

B.2.1.1	General	5153

B.2.1.2	Derivation of Minimum SSB_RP values for FR1	5153

B.2.1.3	Derivation of Minimum SSB_RP values for FR2	5153

B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival	5153

B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage	5154

B.2.1.4	Gain to SS-RSRP and CSI-RSRP measurement point for FR1	5155

B.2.1.5	Gain to SS-RSRP and CSI-RSRP measurement point for FR2	5155

B.2.1.5.1	Gain to SS-RSRP and CSI-RSRP measurement point for Rx Beam Peak angle of arrival	5155

B.2.1.5.2	Gain to SS-RSRP measurement point for different frequency	5156

B.2.1.5.3	Alignment of Rough beam to Rx beam Peak	5156

B.2.1.6	Gain to PRS-RSRP measurement point for FR2	5156

B.2.1.6.1	Gain to PRS-RSRP measurement point for Rx Beam Peak angle of arrival	5156

B.2.2	Conditions for NR intra-frequency measurements	5157

B.2.3	Conditions for NR inter-frequency measurements	5158

B.2.4	Conditions for NR L1-RSRP reporting	5159

B.2.4.1	Conditions for SSB based L1-RSRP reporting	5159

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	5160

B.2.5	Conditions for RRC connection release with redirection to NR	5162

B.2.6	Void	5163

B.2.6.1	Void	5163

B.2.6.2	Void	5163

B.2.7	Conditions for SRS-RSRP measurements	5163

B.2.8	Conditions for NR L1-SINR reporting	5164

B.2.8.1	Conditions for L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	5164

B.2.8.2	Conditions for L1-SINR reporting with SSB based CMR and dedicated IMR configured	5165

B.2.8.2.1	L1-SINR reporting with SSB based CMR and dedicated ZP-IMR configured	5165

B.2.8.2.2	L1-SINR reporting with SSB based CMR and dedicated NZP-IMR configured	5165

B.2.8.3	Conditions for L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	5167

B.2.8.3.1	L1-SINR reporting with CSI-RS based CMR and dedicated ZP-IMR configured	5167

B.2.8.3.2	L1-SINR reporting with CSI-RS based CMR and dedicated NZP-IMR configured	5168

B.2.9	Conditions for NR intra-frequency measurements under CCA	5169

B.2.10	Conditions for NR inter-frequency measurements under CCA	5169

B.2.11	Conditions for NR L1-RSRP reporting under CCA	5169

B.2.11.1	Conditions for SSB based L1-RSRP reporting	5169

B.2.12	Conditions for NR CSI-RS based intra-frequency measurements	5170

B.2.13	Conditions for NR CSI-RS based inter-frequency measurements	5171

B.2.14	Conditions for NR PRS-based measurements	5173

B.2.15	Conditions for NR intra-frequency measurements	5174

B.2.16	Conditions for NR inter-frequency measurements	5176

B.2.17	Conditions for NR intra-frequency measurements for satellite access	5178

B.2.18	Conditions for NR inter-frequency measurements for satellite access	5179

B.2.19	Conditions for NR L1-RSRP reporting for satellite access	5179

B.2.19.1	Conditions for SSB based L1-RSRP reporting for satellite access	5179

B.2.19.2	Conditions for CSI-RS based L1-RSRP reporting for satellite access	5179

B.2.20	Conditions for RRC connection release with redirection to NR for satellite access	5180

B.3	RRM Requirements Exceptions	5180

B.3.1	Introduction	5180

B.3.2	Receiver sensitivity relaxation for CA	5180

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	5180

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	5180

B.3.2.2.1	Inter-band carrier aggregation	5180

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	5180

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	5181

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	5181

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	5181

B.3.2.4.1	Intra-band contiguous carrier aggregation	5181

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	5181

B.3.3	Receiver sensitivity relaxation for DC	5181

B.3.3.1	Receiver sensitivity relaxation for EN-DC	5181

B.3.3.2	Receiver sensitivity relaxation for NE-DC	5181

B.3.4	Receiver sensitivity relaxation for SUL	5181

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	5181

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	5182

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	5182

B.4	Conditions for V2X	5182

B.4.1	Test parameters for GNSS signals	5182

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	5182

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	5183

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	5183

Annex C (informative): Change history	5184