# A.6	NR standalone tests with all NR cells in FR1

## A.6.1	SA: RRC_IDLE state mobility

### A.6.1.1	Cell re-selection to NR

#### A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case

##### A.6.1.1.1.1	Test Purpose and Environment

This test is to verify the requirement for the intra frequency NR cell reselection requirements specified in clause 4.2.2.3.

##### A.6.1.1.1.2	Test Parameters

The test scenario comprises of 1 NR carrier and 2 cells as given in tables A.6.1.1.1.2-1, A.6.1.1.1.2-2 and A.6.1.1.1.2-3. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2.

Table A.6.1.1.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.1.1.1.2-2: General test parameters for intra frequency NR cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell1 |  |
| T2 end condition | Active cell |  | 1, 2, 3 | Cell2 |  |
|  | Neighbour cells |  | 1, 2, 3 | Cell1 |  |
| Final condition | Active cell |  | 1, 2, 3 | Cell1 |  |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC.2 | Configured in SIB2 of Cell 1 |
|  |  |  |  | SMTC.6 | Configured in SIB2 of Cell 2 |
|  |  |  | 2 | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2, 3 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3 | Not configured |  |
| T1 |  | s | 1, 2, 3 | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 1, 2, 3 | 40 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 1, 2, 3 | 15 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.6.1.1.1.2-3: Cell specific test parameters for intra frequency NR cell re-selection test case in AWGN

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1 | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC |  | 1 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
| configuration |  | 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET |  | 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET |  | 1 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -130 |  |  | -130 |  |  |
|  |  | 3 | -127 |  |  | -127 |  |  |
| Pcompensation | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |
| Qhysts | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3 | SS-RSRP |  |  | SS-RSRP |  |  |
|  | dB | 1 | 16 | -3.11 | 2.79 | -infinity | 2.79 | -3.11 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
|  | dB | 1 | 16 | 13 | 16 | -infinity | 16 | 13 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -82 | -85 | -82 | -infinity | -82 | -85 |
|  |  | 2 | -82 | -85 | -82 | -infinity | -82 | -85 |
|  |  | 3 | -79 | -82 | -79 | -infinity | -79 | -82 |
| Io | dBm/9.36 MHz | 1 | -53.94 | -52.21 | -52.21 | Same as parameters specified in Cell 1 columns- |  |  |
|  | dBm/9.36 MHz | 2 | -53.94 | -52.21 | -52.21 |  |  |  |
|  | dBm/38.16 MHz | 3 | -47.85 | -46.12 | -46.12 |  |  |  |
| Treselection | s | 1, 2, 3 | 0 | 0 | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 1, 2, 3 | 60 |  |  | 60 |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

##### A.6.1.1.1.3	Test Requirements

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on Cell 2.

The cell re-selection delay to a newly detectable cell shall be less than 34 s.

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on cell 1.

The cell re-selection delay to an already detected cell shall be less than 8 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as: Tdetect, NR_Intra + TSI-NR, and to an already detected cell can be expressed as: Tevaluate, NR_ intra + TSI-NR,

Where:

Tdetect, NR_Intra	See Table 4.2.2.3-1 in clause 4.2.2.3

Tevaluate, NR_ intra	See Table 4.2.2.3-1 in clause 4.2.2.3

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280ms is assumed in this test case.

This gives a total of 33.28 s, allow 34 s for the cell re-selection delay to a newly detectable cell and 7.68 s for the cell re-selection delay to an already detected cell in the test case, which we allow 8 s.

#### A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case

##### A.6.1.1.2.1	Test Purpose and Environment

This test is to verify the requirement for the inter frequency NR cell reselection requirements specified in clause 4.2.2.4.

##### A.6.1.1.2.2	Test Parameters

The test scenario comprises of 2 cells on 2 different NR carriers respectively as given in tables A.6.1.1.2.2-1, A.6.1.1.2.2-2 and A.6.1.1.2.2-3. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas and cell 2 is of higher priority than cell 1.

Table A.6.1.1.2.2-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.6.1.1.2.2-2: General test parameters for FR1 inter frequency NR cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell2 | The UE camps on cell 2 in the initial phase and during T1 period the UE reselects to cell 1 |
|  | Neighbour cell |  | 1, 2, 3 | Cell 1 |  |
| T1 end condition | Active cell |  | 1, 2, 3 | Cell1 | The UE shall perform reselection to cell 1 during T1 |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| T3 end condition | Active cell |  | 1, 2, 3 | Cell2 | The UE shall perform reselection to cell 2 with higher priority during T3 |
|  | Neighbour cell |  | 1, 2, 3 | Cell 1 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1, 2 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC.2 | Configured in SIB4 of Cell 1 |
|  |  |  |  | SMTC.6 | Configured in SIB4 of Cell 2 |
|  |  |  | 2 | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2, 3 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3 | Not configured |  |
| T1 |  | s | 1, 2, 3 | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3 | >7 | During T2, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T3. |
| T3 |  | s | 1, 2, 3 | 75 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.6.1.1.2.2-3: Cell specific test parameters for FR1 inter frequency NR cell re-selection test case in AWGN

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1 | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC |  | 1 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
| configuration |  | 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET |  | 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET |  | 1 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -140 |  |  | -140 |  |  |
|  |  | 3 | -137 |  |  | -137 |  |  |
| Pcompensation | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3 | SS-RSRP |  |  | SS-RSRP |  |  |
|  | dB | 1 | 14 | 14 | 14 | -4 | -infinity | 12 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
|  | dB | 1 | 14 | 14 | 14 | -4 | -infinity | 12 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -84 | -84 | -84 | -102 | -infinity | -86 |
|  |  | 2 | -84 | -84 | -84 | -102 | -infinity | -86 |
|  |  | 3 | -81 | -81 | -81 | -99 | -infinity | -83 |
| Io | dBm/9.36 MHz | 1 | -55.88 | -55.88 | -55.88 | -68.60 | -70.05 | -57.78 |
|  | dBm/9.36 MHz | 2 | -55.88 | -55.88 | -55.88 | -68.60 | -70.05 | -57.78 |
|  | dBm/38.16 MHz | 3 | -49.79 | -49.79 | -49.79 | -62.50 | -63.96 | -51.69 |
| Treselection | s | 1, 2, 3 | 0 | 0 | 0 | 0 | 0 | 0 |
| SnonintrasearchP | dB | 1, 2, 3 | 50 |  |  | 50 |  |  |
| Threshx, highP | dB | 1, 2, 3 | 48 |  |  | 48 |  |  |
| Threshserving, lowP | dB | 1, 2, 3 | 44 |  |  | 44 |  |  |
| Threshx, lowP | dB | 1, 2, 3 | 50 |  |  | 50 |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

##### A.6.1.1.2.3	Test Requirements

The cell reselection delay to a higher priority cell is defined as the time from the beginning of time period T3, to the moment when the UE camps again on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on cell 2.

The cell re-selection delay to a higher priority cell shall be less than 68 s.

The cell reselection delay to a lower priority cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on cell 1.

The cell re-selection delay to a lower priority cell shall be less than 8 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a higher priority cell can be expressed as: Thigher_priority_search + Tevaluate, NR_ inter + TSI-NR, and to a lower priority cell can be expressed as: Tevaluate, NR_ inter + TSI-NR,

Where:

Thigher_priority_search	See clause 4.2.2.7

Tevaluate, NR_ inter	See Table 4.2.2.4-1 in clause 4.2.2.4

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 67.68 s, allow 68 s for the cell re-selection delay to a higher priority cell and 7.68 s for the cell re-selection delay to a lower priority cell in the test case, which we allow 8 s.

#### A.6.1.1.3	Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion

##### A.6.1.1.3.1	Test Purpose and Environment

This test is to verify the requirement for the intra frequency NR cell reselection requirements for UE fulfilling low mobility criterion specified in clause 4.2.2.9.2

##### A.6.1.1.3.2	Test Parameters

The test scenario comprises of 1 NR carrier and 2 cells as given in tables A.6.1.1.3.2-1, A.6.1.1.3.2-2 and A.6.1.1.3.2-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2.

Table A.6.1.1.3.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.1.1.3.2-2: General test parameters for FR1 intra frequency NR cell re-selection test case for UE fulfilling low mobility criterion

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell1 | The UE camps on cell 1 in the initial phase |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| T1 end condition | Active cell |  | 1, 2, 3 | Cell2 | The UE reselects to cell 2 during T1 period |
|  | Neighbour cells |  | 1, 2, 3 | Cell1 |  |
| Final condition | Active cell |  | 1, 2, 3 | Cell1 | The UE reselects to cell 1 during T2 period |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC pattern 2 | Configured in SIB2 of Cell 1 |
|  |  |  |  | SMTC pattern 6 | Configured in SIB2 of Cell 2 |
|  |  |  | 2 | SMTC pattern 1 |  |
|  |  |  | 3 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1, 2, 3 | 0.64 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3 | Not configured |  |
| T1 |  | s | 1, 2, 3 | 25 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3 | 25 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.6.1.1.3.2-3: Cell specific test parameters for FR1 intra frequency NR cell re-selection test case in AWGN for UE fulfilling low mobility criterion

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  | OP.1 defined in A.3.2.1 |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -140 |  | -140 |  |
|  |  | 3 | -137 |  | -137 |  |
| Pcompensation | dB | 1, 2, 3 | 0 |  | 0 |  |
| Qhysts | dB | 1, 2, 3 | 0 |  | 0 |  |
| Qoffsets, n | dB | 1, 2, 3 | 0 |  | 0 |  |
| SSearchDeltaP | dB | 1, 2, 3 | 3 |  | 3 |  |
| TSearchDeltaP | s | 1, 2, 3 | 5 |  | 5 |  |
| Cell_selection_and_ reselection_quality_ measurement |  | 1, 2, 3 | SS-RSRP |  | SS-RSRP |  |
|  | dB | 1, 2, 3 | -3.11 | 2.79 | 2.79 | -3.11 |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note2 | dBm/15 kHz | 1, 2, 3 | -98 |  |  |  |
|  | dB | 1, 2, 3 | 13 | 16 | 16 | 13 |
| SS-RSRP Note3 | dBm/SCS | 1 | -85 | -82 | -82 | -85 |
|  |  | 2 | -85 | -82 | -82 | -85 |
|  |  | 3 | -82 | -79 | -79 | -82 |
| Io | dBm/9.36 MHz | 1 | -52.21 | -52.21 | specified in Cell 1 columns- |  |
|  | dBm/9.36 MHz | 2 | -52.21 | -52.21 |  |  |
|  | dBm/38.16 MHz | 3 | -46.12 | -46.12 |  |  |
| Treselection | s | 1, 2, 3 | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 1, 2, 3 | 60 |  | 60 |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.1.1.3.3	Test Requirements

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to an already detected cell shall be less than 17 s.

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 1.

The cell re-selection delay to an already detected cell shall be less than 17 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to an already detected cell can be expressed as: Tevaluate,NR_Intra + TSI-NR,

Where:

Tevaluate,NR_Intra	See Table 4.2.2.9.2-1 in clause 4.2.2.9.2 for reselection to Cell 2 during T1 with UE fulfilling low mobility criterion. 15.36s.

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 16.64 s, allow 17 s for the cell re-selection delay to an already detected cell for UE fulfilling low mobility criterion in the test case.

#### A.6.1.1.4	Cell reselection to FR1 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion

##### A.6.1.1.4.1	Test Purpose and Environment

This test is to verify the relaxed cell re-selection requirement for UEs configured with not-at-cell edge criterion specified in clause 4.2.2.9.3.

##### A.6.1.1.4.2	Test Parameters

The test scenario comprises of 1 NR carrier and 2 cells as given in tables A.6.1.1.4.2-1, A.6.1.1.4.2-2 and A.6.1.1.4.2-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both Cell 1 and Cell 2 are already identified by the UE prior to the start of the test. Cell 1 and Cell 2 belong to different tracking areas.

Table A.6.1.1.4.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.1.1.4.2-2: General test parameters for FR1 intra frequency NR cell re-selection test case for UE fulfilling not-at-cell edge criterion

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active Cell |  | 1, 2, 3 | Cell1 | The UE camps on Cell 1 in the initial phase |
|  | Neighbour Cells |  | 1, 2, 3 | Cell2 |  |
| T1 end condition | Active Cell |  | 1, 2, 3 | Cell2 | The UE shall fulfil the not-at-cell edge criterion and reselect to cell 2 during T1 period during T1. |
|  | Neighbour Cells |  | 1, 2, 3 | Cell1 |  |
| T2 end condition | Active Cell |  | 1, 2, 3 | Cell1 | The UE shall perform reselection to Cell 1 during T2 |
|  | Neighbour Cells |  | 1, 2, 3 | Cell2 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1 |  |
| Time offset between Cells |  |  | 1 | 3 ms | Asynchronous Cells |
|  |  |  | 2 | 3 s | Synchronous Cells |
|  |  |  | 3 | 3 s | Synchronous Cells |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC pattern 2 | Configured in SIB2 of Cell 1 |
|  |  |  |  | SMTC pattern 6 | Configured in SIB2 of Cell 2 |
|  |  |  | 2 | SMTC pattern 1 |  |
|  |  |  | 3 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1, 2, 3 | 0.64 | The value shall be used for all Cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3 | Not configured |  |
| T1 |  | s | 1, 2, 3 | 20 | T1 needs to be defined so that Cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3 | 20 | T2 needs to be defined so that Cell re-selection reaction time is taken into account. |

Table A.6.1.1.4.2-3: Cell specific test parameters for FR1 intra frequency NR cell re-selection test case in AWGN for UE fulfilling not-at-cell edge criterion

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  | OP.1 defined in A.3.2.1 |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -140 |  | -140 |  |
|  |  | 3 | -137 |  | -137 |  |
| Pcompensation | dB | 1, 2, 3 | 0 |  | 0 |  |
| Qhysts | dB | 1, 2, 3 | 0 |  | 0 |  |
| Qoffsets, n | dB | 1, 2, 3 | 0 |  | 0 |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3 | SS-RSRP |  | SS-RSRP |  |
|  | dB | 1 | -3.11 | 2.79 | 2.79 | -3.11 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 13 | 16 | 16 | 13 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -85 | -82 | -82 | -85 |
|  |  | 2 | -85 | -82 | -82 | -85 |
|  |  | 3 | -82 | -79 | -79 | -82 |
| Io | dBm/9.36 MHz | 1 | -52.21 | -52.21 | -52.21 | -52.21 |
|  | dBm/9.36 MHz | 2 | -52.21 | -52.21 | -52.21 | -52.21 |
|  | dBm/38.16 MHz | 3 | -46.12 | -46.12 | -46.12 | -46.12 |
| Treselection | s | 1, 2, 3 | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 1, 2, 3 | 60 |  | 60 |  |
| SsearchThresholdP | dB | 1, 2, 3 | 50 | Not sent | Not sent | 50 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both Cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other Cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.1.1.4.3	Test Requirements

The cell re-selection delay to an already detected cell for UE configured with cellEdgeEvaluation criterion is defined as the time from the beginning of time period T1, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to an already detected cell for UE configured with cellEdgeEvaluation criterion shall be less than 17s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to an already detected cell for UE configured with relaxed measurement criterion can be expressed as: Tevaluate,NR_Intra + TSI-NR,

Where:

Tevaluate,NR_Intra	See Table 4.2.2.9.3-1 for UE fulfilling not-at-cell edge criterion in clause 4.2.2.9.3.

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a Cell; 1280ms is assumed in this test case.

This gives a total of 16.64s, allow 17s for the cell re-selection delay to an already detected cell for UE fulfilling not-at-cell edge criterion in the test case.

#### A.6.1.1.5	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion

##### A.6.1.1.5.1	Test Purpose and Environment

This test is to verify the requirement for the inter frequency NR cell reselection requirements specified in clause 4.2.2.10.2, for UE fulfilling low mobility relaxed measurement criterion.

##### A.6.1.1.5.2	Test Parameters

The test scenario comprises of 2 cells on 2 different NR carriers respectively as given in tables A.6.1.1.5.2-1, A.6.1.1.5.2-2 and A.6.1.1.5.2-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas and cell 2 is of higher priority than cell 1.

As specified in the Test Purpose, the UE is configured with the relaxed measurement criterion for UE with low mobility defined in clause 5.2.4.9.1 in [1]. So, Cell 2 and Cell 1 configure the UE as follows:

lowMobilityEvalutation [2] criterion is configured according to the parameters listed in Table A.6.1.1.5.2-3;

cellEdgeEvaluation [2] criterion is not configured;

combineRelaxedMeasCondition [2] is not configured;

Table A.6.1.1.5.2-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.6.1.1.5.2-2: General test parameters for FR1 inter frequency NR cell re-selection test case for UE fulfilling low mobility criterion

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell2 | The UE camps on cell 2 in the initial phase, it fulfills Low Mobility relaxation measurements criterion, and during T1 period the UE reselects to cell 1 |
|  | Neighbour cells |  | 1, 2, 3 | Cell1 |  |
| T1 end condition | Active cell |  | 1, 2, 3 | Cell1 | The UE shall perform reselection to cell 1 during T1 |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| T2 end condition | Active cell |  | 1, 2, 3 | Cell2 | The UE shall perform reselection to cell 2 with higher priority during T2 |
|  | Neighbour cells |  | 1, 2, 3 | Cell1 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1, 2 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB Configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC pattern 2 | Configured in SIB4 of Cell 1 |
|  |  |  |  | SMTC pattern 6 | Configured in SIB4 of Cell 2 |
|  |  |  | 2 | SMTC pattern 1 |  |
|  |  |  | 3 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1, 2, 3 | 0.64 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3 | Not configured |  |
| T1 |  | s | 1, 2, 3 | 25 s | T1 is defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3 | 25 s | T2 is defined so that cell re-selection reaction time is taken into account. |

Table A.6.1.1.5.2-3: Cell specific test parameters for FR1 inter frequency NR cell re-selection test case in AWGN for UE fulfilling low mobility criterion

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 |  | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |  |
| PDSCH RMC |  | 1 | SR.1.1 FDD |  | SR.1.1 FDD |  |  |
| configuration |  | 2 | SR.1.1 TDD |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  | SR.2.1 TDD |  |  |
| RMSI CORESET |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |  |
| Dedicated CORESET |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -140 |  | -140 |  |  |
|  |  | 3 | -137 |  | -137 |  |  |
| Pcompensation | dB | 1, 2, 3 | 0 |  | 0 |  |  |
| Qhysts | dB | 1, 2, 3 | 0 |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2, 3 | 0 |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3 | SS-RSRP |  | SS-RSRP |  |  |
|  | dB | 1 | 14 | 14 | -4 | 12 |  |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
|  | dB | 1 | 14 | 14 | -4 |  | 12 |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -84 | -84 | -102 |  | -86 |
|  |  | 2 | -84 | -84 | -102 |  | -86 |
|  |  | 3 | -81 | -81 | -99 |  | -83 |
| Io | dBm/9.36 MHz | 1 | -55.88 | -55.88 | -68.60 |  | -57.78 |
|  | dBm/9.36 MHz | 2 | -55.88 | -55.88 | -68.60 |  | -57.78 |
|  | dBm/38.16 MHz | 3 | -49.79 | -49.79 | -62.50 |  | -51.69 |
| Treselection | s | 1, 2, 3 | 0 | 0 | 0 |  | 0 |
| SnonintrasearchP | dB | 1, 2, 3 | Not sent |  | Not sent |  |  |
| Threshx, highP | dB | 1, 2, 3 | 48 |  | 48 |  |  |
| Threshserving, lowP | dB | 1, 2, 3 | 44 |  | 44 |  |  |
| Threshx, lowP | dB | 1, 2, 3 | 50 |  | 50 |  |  |
| SSearchDeltaP | dB | 1, 2, 3 | 3 |  | 3 |  |  |
| TSearchDeltaP | s | 1, 2, 3 | 5 |  | 5 |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

##### A.6.1.1.5.3	Test Requirements

The cell reselection delay to an already detected lower priority cell for UE fulfilling low mobility relaxed measurements is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 1.

The cell re-selection delay to a lower priority cell for UE fulfilling low mobility relaxed measurements shall be less than 17 s.

The cell reselection delay to an already detected higher priority cell for UE fulfilling low mobility relaxed measurements is defined as the time from the beginning of time period T2, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to an already detected higher priority cell for UE fulfilling low mobility relaxed measurements shall be less than 17 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a known lower priority cell can be expressed as: Tevaluate, NR_ inter + TSI-NR,

Where:

Tevaluate, NR_ inter	See Table 4.2.2.10.2-1 in clause 4.2.2.10.2

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 16.64 s, allow 17s  for the cell re-selection delay to an already detected lower priority cell and 16.64s for the cell re-selection delay to an already detected higher priority cell, which we allow 17s for UE fulfilling low mobility relaxed measurements in the test case.

#### A.6.1.1.6	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion

##### A.6.1.1.6.1	Test Purpose and Environment

This test is to verify the requirement for the inter frequency NR cell reselection requirements specified in clause 4.2.2.10.3, for UE fulfilling not-at-cell edge relaxed measurement criterion.

##### A.6.1.1.6.2	Test Parameters

The test scenario comprises of 2 cells on 2 different NR carriers respectively as given in tables A.6.1.1.6.2-1, A.6.1.1.6.2-2 and A.6.1.1.6.2-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas and cell 2 is of higher priority than cell 1.

As specified in the Test Purpose, the UE is configured with the relaxed measurement criterion for UE not-at-cell edge as defined in clause 5.2.4.9.2 in [1]. So, Cell 2 and Cell 1configures the UE as follows:

cellEdgeEvaluation [2] criterion is configured according to the parameters listed in Table A.6.1.1.5.2-3;

lowMobilityEvalutation [2] criterion is not configured;

combineRelaxedMeasCondition [2] is not configured;

Table A.6.1.1.6.2-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.6.1.1.6.2-2: General test parameters for FR1 inter frequency NR cell re-selection test case for UE fulfilling not-at-cell edge criterion

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell2 | The UE camps on cell 2 in the initial phase, it fulfills Not-at-cell edge relaxation measurements criterion, and during T1 period the UE reselects to cell 1 |
|  | Neighbour cells |  | 1, 2, 3 | Cell1 |  |
| T1 end condition | Active cell |  | 1, 2, 3 | Cell1 | The UE shall perform reselection to cell 1 during T1 |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| T2 end condition | Active cell |  | 1, 2, 3 | Cell2 | The UE shall perform reselection to cell 2 with higher priority during T2 |
|  | Neighbour cells |  | 1, 2, 3 | Cell1 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1, 2 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
|  |  |  | 1 | SSB.1 FR1 |  |
| SSB Configuration |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC pattern 2 | Configured in SIB4 of Cell 1 |
|  |  |  |  | SMTC pattern 6 | Configured in SIB4 of Cell 2 |
|  |  |  | 2 | SMTC pattern 1 |  |
|  |  |  | 3 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1, 2, 3 | 0.64 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3 | Not configured |  |
| T1 |  | s | 1, 2, 3 | 20 s | T1 is defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3 | 20 s | T2 is defined so that cell re-selection reaction time is taken into account. |

Table A.6.1.1.6.2-3: Cell specific test parameters for FR1 inter frequency NR cell re-selection test case in AWGN for UE fulfilling not-at-cell edge criterion

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 |  | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |  |
| PDSCH RMC |  | 1 | SR.1.1 FDD |  | SR.1.1 FDD |  |  |
| configuration |  | 2 | SR.1.1 TDD |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  | SR.2.1 TDD |  |  |
| RMSI CORESET |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |  |
| Dedicated CORESET |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -140 |  | -140 |  |  |
|  |  | 3 | -137 |  | -137 |  |  |
| Pcompensation | dB | 1, 2, 3 | 0 |  | 0 |  |  |
| Qhysts | dB | 1, 2, 3 | 0 |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2, 3 | 0 |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3 | SS-RSRP |  | SS-RSRP |  |  |
|  | dB | 1 | 14 | 14 | -4 | 12 |  |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
|  | dB | 1 | 14 | 14 | -4 |  | 12 |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -84 | -84 | -102 |  | -86 |
|  |  | 2 | -84 | -84 | -102 |  | -86 |
|  |  | 3 | -81 | -81 | -99 |  | -83 |
| Io | dBm/9.36 MHz | 1 | -55.88 | -55.88 | -68.60 |  | -57.78 |
|  | dBm/9.36 MHz | 2 | -55.88 | -55.88 | -68.60 |  | -57.78 |
|  | dBm/38.16 MHz | 3 | -49.79 | -49.79 | -62.50 |  | -51.69 |
| Treselection | s | 1, 2, 3 | 0 | 0 | 0 |  | 0 |
| SnonintrasearchP | dB | 1, 2, 3 | Not sent |  | Not sent |  |  |
| Threshx, highP | dB | 1, 2, 3 | 48 |  | 48 |  |  |
| Threshserving, lowP | dB | 1, 2, 3 | 44 |  | 44 |  |  |
| Threshx, lowP | dB | 1, 2, 3 | 50 |  | 50 |  |  |
| SSearchThresholdP | dB | 1, 2, 3 | 50 |  | 50 |  |  |
| SSearchThresholdQ | s | 1, 2, 3 | Not Configured |  |  |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

##### A.6.1.1.6.3	Test Requirements

The cell reselection delay to an already detected lower priority cell for UE fulfilling not-at-cell edge relaxed measurements is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 1.

The cell re-selection delay to an already detected lower priority cell for UE fulfilling not-at-cell edge relaxed measurements shall be less than 17 s.

The cell reselection delay to an already detected higher priority cell for UE fulfilling not-at-cell-edge relaxed measurements is defined as the time from the beginning of time period T2, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to an already detected higher priority cell for UE fulfilling not-at-cell-edge  relaxed measurements shall be less than 17 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a lower priority cell can be expressed as: Tevaluate, NR_ inter + TSI-NR,

Where:

Tevaluate, NR_ inter	See Table 4.2.2.10.3-1 in clause 4.2.2.10

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 16.64 s , allow 17s for the cell re-selection delay to an already detected lower priority cell and 16.64s for the cell re-selection delay to an already higher priority cell, which we allow 17s  for UE fulfilling not-at-cell edge relaxed measurements in the test case.

#### A.6.1.1.7	Cell reselection to FR1 intra-frequency NR case for UE configured with highSpeedMeasFlag-r16

##### A.6.1.1.7.1	Test Purpose and Environment

This test is to verify the requirement for the intra frequency NR cell reselection requirements for UE configured with highSpeedMeasFlag-r16 specified in clause 4.2.2.3.

##### A.6.1.1.7.2	Test Parameters

The test scenario comprises of 1 NR carrier and 2 cells as given in tables A.6.1.1.1.x-1, A.6.1.1.1.x-2 and A.6.1.1.1.x-3. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2. highSpeedMeasFlag-r16 is broadcasted to UE.

Table A.6.1.1.7.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.1.1.7.2-2: General test parameters for intra frequency NR cell re-selection test case for UE configured with highSpeedMeasFlag-r16

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial | Active cell |  | 1, 2, 3 | Cell1 |  |
| condition | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| T2 end condition | Active cell |  | 1, 2, 3 | Cell2 |  |
|  | Neighbour cells |  | 1, 2, 3 | Cell1 |  |
| Final condition | Active cell |  | 1, 2, 3 | Cell1 |  |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC pattern 2 | Configured in SIB2 of Cell 1 |
|  |  |  |  | SMTC pattern 6 | Configured in SIB2 of Cell 2 |
|  |  |  | 2 | SMTC pattern 1 |  |
|  |  |  | 3 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1, 2, 3 | 0.32 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3 | Not configured |  |
| T1 |  | s | 1, 2, 3 | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 1, 2, 3 | 4 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 1, 2, 3 | 3 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.6.1.1.7.2-3: Cell specific test parameters for intra frequency NR cell re-selection test case for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |  | T1 | T2 | T3 |
| TDD configuration |  | 1 | N/A |  |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC |  | 1 | SR.1.1 FDD |  |  |  | SR.1.1 FDD |  |  |
| configuration |  | 2 | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET |  | 1 | CR.1.1 FDD |  |  |  | CR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET |  | 1 | CCR.1.1 FDD |  |  |  | CCR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -140 |  |  |  | -140 |  |  |
|  |  | 3 | -137 |  |  |  | -137 |  |  |
| Pcompensation | dB | 1, 2, 3 | 0 |  |  |  | 0 |  |  |
| Qhysts | dB | 1, 2, 3 | 0 |  |  |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2, 3 | 0 |  |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3 | SS-RSRP |  |  |  | SS-RSRP |  |  |
|  | dB | 1 | 16 | -3.11 | 2.79 |  | -infinity | 2.79 | -3.11 |
|  |  | 2 |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |
|  | dB | 1 | 16 | 13 | 16 |  | -infinity | 16 | 13 |
|  |  | 2 |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -82 | -85 | -82 |  | -infinity | -82 | -85 |
|  |  | 2 | -82 | -85 | -82 |  | -infinity | -82 | -85 |
|  |  | 3 | -79 | -82 | -79 |  | -infinity | -79 | -82 |
| Io | dBm/9.36 MHz | 1 | -53.94 | -52.21 | -52.21 |  | Same as parameters specified in Cell 1 columns- |  |  |
|  | dBm/9.36 MHz | 2 | -53.94 | -52.21 | -52.21 |  |  |  |  |
|  | dBm/38.16 MHz | 3 | -47.85 | -46.12 | -46.12 |  |  |  |  |
| Treselection | s | 1, 2, 3 | 0 | 0 | 0 |  | 0 | 0 | 0 |
| SintrasearchP | dB | 1, 2, 3 | 60 |  |  |  | 60 |  |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  | AWGN 1944Hz Note4 |  |  |  |
| Propagation Condition |  | 3 | AWGN |  |  | AWGN 3334Hz Note5 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The AWGN 1944 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 1944 Hz. Note 5: The AWGN 3334 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 3334 Hz. |  |  |  |  |  |  |  |  |  |

##### A.6.1.1.7.3	Test Requirements

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to a newly detectable cell shall be less than 4 s.

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 1.

The cell re-selection delay to an already detected cell shall be less than 3 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as: Tdetect, NR_Intra + TSI-NR, and to an already detected cell can be expressed as: Tevaluate, NR_ intra + TSI-NR,

Where:

Tdetect, NR_Intra	See Table 4.2.2.3-2 in clause 4.2.2.3

Tevaluate, NR_ intra	See Table 4.2.2.3-2 in clause 4.2.2.3

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280ms is assumed in this test case.

This gives a total of 3.84s, allow 4s for the cell re-selection delay to a newly detectable cell and 2.24 s for the cell re-selection delay to an already detected cell in the test case, which we allow 3 s.

#### A.6.1.1.8	Cell reselection to FR1 inter-frequency NR case for UE configured with highSpeedMeasInterFreq-r17

##### A.6.1.1.8.1	Test Purpose and Environment

This test is to verify the requirement for the inter frequency NR cell reselection requirements for UE configured with highSpeedMeasInterFreq-r17 specified in clause 4.2.2.4.

##### A.6.1.1.8.2	Test Parameters

The test scenario comprises of 2 cells on 2 different NR carriers respectively as given in tables A.6.1.1.8.2-1, A.6.1.1.8.2-2 and A.6.1.1.8.2-3. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas.

Table A.6.1.1.8.2-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.6.1.1.8.2-2: General test parameters for FR1 inter frequency NR cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell2 | The UE camps on cell 2 in the initial phase and during T1 period the UE reselects to cell 1 |
|  | Neighbour cell |  | 1, 2, 3 | Cell 1 |  |
| T1 end condition | Active cell |  | 1, 2, 3 | Cell1 | The UE shall perform reselection to cell 1 during T1 |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| T3 end condition | Active cell |  | 1, 2, 3 | Cell2 | The UE shall perform reselection to cell 2 during T3 |
|  | Neighbour cell |  | 1, 2, 3 | Cell 1 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1, 2 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC.2 |  |
|  |  |  | 2 | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2, 3 | 0.32 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3 | Not configured |  |
| T1 |  | s | 1, 2, 3 | 10 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3 | >7 | During T2, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T3. |
| T3 |  | s | 1, 2, 3 | 12 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.6.1.1.8.2-3: Cell specific test parameters for FR1 inter frequency NR cell re-selection test case in AWGN

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1 | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC |  | 1 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
| configuration |  | 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET |  | 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET |  | 1 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -140 |  |  | -140 |  |  |
|  |  | 3 | -137 |  |  | -137 |  |  |
| Pcompensation | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3 | SS-RSRP |  |  | SS-RSRP |  |  |
|  | dB | 1 | 14 | 14 | 14 | -4 | -infinity | 12 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
|  | dB | 1 | 14 | 14 | 14 | -4 | -infinity | 12 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -84 | -84 | -91 | -102 | -infinity | -86 |
|  |  | 2 | -84 | -84 | -84 | -102 | -infinity | -86 |
|  |  | 3 | -81 | -81 | -81 | -99 | -infinity | -83 |
| Io | dBm/9.36 MHz | 1 | -55.88 | -55.88 | -55.88 | -68.60 | -70.05 | -57.78 |
|  | dBm/9.36 MHz | 2 | -55.88 | -55.88 | -55.88 | -68.60 | -70.05 | -57.78 |
|  | dBm/38.16 MHz | 3 | -49.79 | -49.79 | -49.79 | -62.50 | -63.96 | -51.69 |
| Treselection | s | 1, 2, 3 | 0 | 0 | 0 | 0 | 0 | 0 |
| SnonintrasearchP | dB | 1, 2, 3 | 50 |  |  | 50 |  |  |
| Threshx, highP | dB | 1, 2, 3 | 48 |  |  | 48 |  |  |
| Threshserving, lowP | dB | 1, 2, 3 | 44 |  |  | 44 |  |  |
| Threshx, lowP | dB | 1, 2, 3 | 50 |  |  | 50 |  |  |
| Propagation Condition |  | 1, 2 | AWGN for active cell, AWGN 1944HzNote 4 for neighbour cell |  |  |  |  |  |
| Propagation Condition |  | 3 | AWGN for active cell, AWGN 3334HzNote 5 for neighbour cell |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The AWGN 1944 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 1944 Hz. Note 5: The AWGN 3334 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 3334 Hz. |  |  |  |  |  |  |  |  |

##### A.6.1.1.8.3	Test Requirements

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to a newly detectable cell shall be less than 5 s.

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 1.

The cell re-selection delay to an already detected cell shall be less than 3 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as: Tdetect,NR_Inter_HST + TSI-NR, and to an already detected cell can be expressed as: Tevaluate,NR_Inter_HST + TSI-NR,

Where:

Tdetect,NR_Inter_HST See Table 4.2.2.4-2 in clause 4.2.2.4

Tevaluate,NR_Inter_HST	 See Table 4.2.2.4-2 in clause 4.2.2.4

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280ms is assumed in this test case.

This gives a total of 4.48s, allow 5s for the cell re-selection delay to a newly detectable cell and 2.24 s for the cell re-selection delay to an already detected cell in the test case, which we allow 3 s.

### A.6.1.2	Inter-RAT E-UTRAN cell re-selection

#### A.6.1.2.1	Cell reselection to higher priority E-UTRAN

##### A.6.1.2.1.1	Test Purpose and Environment

This test is to verify the requirement for the NR to E-UTRAN inter-RAT cell reselection requirements specified in clause 4.2.2.5 when the E-UTRAN cell is of higher priority.

##### A.6.1.2.1.2	Test Parameters

The test scenario comprises of one NR cell and one E-UTRAN cell as given in tables A.6.1.2.1.2-1, A.6.1.2.1.2-2, A.6.1.2.1.2-3 and A.6.1.2.1.2-4. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. NR cell 1 is already identified by the UE prior to the start of the test. E-UTRAN cell 2 is of higher priority than cell 1.

Table A.6.1.2.1.2-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.6.1.2.1.2-2: General test parameters for NR to E-UTRAN cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE camps on cell 1 in the initial phase and during T2 period the UE reselects to cell 2. |
| T2 end | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell2 | The UE shall perform reselection to cell |
| condition | Neighbour cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | 2 during T2. |
| T3 end | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE shall perform reselection to cell |
| condition | Neighbour cell |  | 1, 2, 3, 4, 5, 6 | Cell2 | 1 during T3 for iteration of the tests. |
| Access Barring Information |  | - | 1, 2, 3, 4, 5, 6 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2, 3, 4, 5, 6 | 1.28 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2, 3, 4, 5, 6 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| E-UTRAN PRACH configuration index |  |  | 1, 2, 3 | 53 | As specified in table 5.7.1-2 in TS 36.211 [23] |
|  |  |  | 4, 5, 6 | 4 |  |
| E-UTRAN PRACH |  |  | 1, 2, 3 | 53 | As specified in table 5.7.1-2 in |
| configuration index |  |  | 4, 5, 6 | 4 | TS 36.211 [23] |
| T1 |  | s | 1, 2, 3, 4, 5, 6 | >7 | During T1, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T2. |
| T2 |  | s | 1, 2, 3, 4, 5, 6 | 75 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 1, 2, 3, 4, 5, 6 | 15 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.6.1.2.1.2-3: Cell specific test parameters for NR cell 1

| Parameter | Unit | Test configuration | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| TDD configuration |  | 1, 4 | N/A |  |  |
|  |  | 2, 5 | TDDConf.1.1 |  |  |
|  |  | 3, 6 | TDDConf.2.1 |  |  |
| PDSCH parameters |  | 1, 4 | SR.1.1 FDD |  |  |
|  |  | 2, 5 | SR.1.1 TDD |  |  |
|  |  | 3, 6 | SR.2.1 TDD |  |  |
| RMSI CORESET parameters |  | 1, 4 | CR.1.1 FDD |  |  |
|  |  | 2, 5 | CR.1.1 TDD |  |  |
|  |  | 3, 6 | CR.2.1 TDD |  |  |
| Dedicated CORESET parameters |  | 1, 4 | CCR.1.1 FDD |  |  |
|  |  | 2, 5 | CCR.1.1 TDD |  |  |
|  |  | 3, 6 | CCR.2.1 TDD |  |  |
| SSB parameters |  | 1, 4 | SSB.1 FR1 |  |  |
|  |  | 2, 5 | SSB.1 FR1 |  |  |
|  |  | 3, 6 | SSB.2 FR1 |  |  |
| NR SMTC parameters |  | 1, 4 | SMTC.2 |  |  |
|  |  | 2, 5 | SMTC.1 |  |  |
|  |  | 3, 6 | SMTC.1 |  |  |
| OCNG Pattern |  | 1, 2, 3, 4, 5, 6 | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3, 4, 5, 6 | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4, 5 | -140 |  |  |
|  |  | 3, 6 | -137 |  |  |
|  | dBm/SCS | 1, 4 | -98 |  |  |
|  |  | 2, 5 | -98 |  |  |
|  |  | 3, 6 | -95 |  |  |
|  | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -98 |  |  |
| SS-RSRP | dBm/SCS | 1, 4 | -84 | -84 | -84 |
|  |  | 2, 5 | -84 | -84 | -84 |
|  |  | 3, 6 | -81 | -81 | -81 |
|  | dB | 1, 4 | 14 | 14 | 14 |
|  |  | 2, 5 |  |  |  |
|  |  | 3, 6 |  |  |  |
|  | dB | 1, 4 | 14 | 14 | 14 |
|  |  | 2, 5 |  |  |  |
|  |  | 3, 6 |  |  |  |
| Io | dBm/9.36 MHz | 1, 4 | -55.88 | -55.88 | -55.88 |
|  | dBm/9.36 MHz | 2, 5 | -55.88 | -55.88 | -55.88 |
|  | dBm/38.16 MHz | 3, 6 | -49.79 | -49.79 | -49.79 |
| Treselection | S | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| SnonintrasearchP | dB | 1, 2, 3, 4, 5, 6 | 50 |  |  |
| Threshx, highP (Note 2) | dB | 1, 2, 3, 4, 5, 6 | 48 |  |  |
| Threshserving, lowP | dB | 1, 2, 3, 4, 5, 6 | 44 |  |  |
| Threshx, lowP | dB | 1, 2, 3, 4, 5, 6 | 50 |  |  |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, high which is included in NR system information, and is a threshold for the E-UTRA target cell |  |  |  |  |  |

Table A.6.1.2.1.2-4: Cell specific test parameters for E-UTRA cell 2

| Parameter | Unit | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 |  | T3 |
| E-UTRA RF Channel number |  | 1 |  |  |  |
| BWchannel | MHz | 10 |  |  |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Qrxlevmin | dBm | -140 |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |
| RSRP | dBm/15 KHz | -infinity |  | -86 | -102 |
|  | dB | -infinity |  | 12 | -4 |
|  | dB | -infinity |  | 12 | -4 |
| TreselectionEUTRAN | S | 0 |  |  |  |
| SnonintrasearchP | dB | Not sent |  |  |  |
| Threshx, highP | dB | 48 |  |  |  |
| Threshserving, lowP | dB | 44 |  |  |  |
| Threshx, lowP (Note 2) | dB | 50 |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, Low which is included in E-UTRA system information, and is a threshold for the NR target cell |  |  |  |  |  |

##### A.6.1.2.1.3	Test Requirements

The cell reselection delay to a higher priority E-UTRAN cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to a higher priority cell shall be less than 68 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a higher priority cell can be expressed as: Thigher_priority_search + Tevaluate, E-UTRAN + TSI-E-UTRA,

Where:

Thigher_priority_search	See clause 4.2.2.7

Tevaluate, E-UTRAN	See Table 4.2.2.5-1 in clause 4.2.2.5

TSI-E-UTRA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 67.68 s, allow 68 s for the cell re-selection delay to a higher priority E-UTRAN cell.

#### A.6.1.2.2	Cell reselection to lower priority E-UTRAN

##### A.6.1.2.2.1	Test Purpose and Environment

This test is to verify the requirement for the NR to E-UTRAN inter-RAT cell reselection requirements specified in clause 4.2.2.5 when the E-UTRAN cell is of lower priority.

##### A.6.1.2.2.2	Test Parameters

The test scenario comprises of one NR cell and one E-UTRAN cell as given in tables A.6.1.2.2.2-1, A.6.1.2.2.2-2, A.6.1.2.2.2-3 and A.6.1.2.2.2-4. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both NR cell 1 and E-UTRAN cell 2 are already identified by the UE prior to the start of the test. E-UTRAN cell 2 is of lower priority than cell 1.

Table A.6.1.2.2.2-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.6.1.2.2.2-2: General test parameters for NR to E-UTRAN cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE camps on cell 1 in the initial phase. |
|  | Neighbour cell |  | 1, 2, 3, 4, 5, 6 | Cell2 |  |
| T1 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell2 | The UE shall perform reselection to cell 2 during T1. |
|  | Neighbour cell |  | 1, 2, 3, 4, 5, 6 | Cell1 |  |
| T2 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE shall perform reselection to cell 1 during T2 for iteration of the tests. |
|  | Neighbour cell |  | 1, 2, 3, 4, 5, 6 | Cell2 |  |
| Access Barring Information |  | - | 1, 2, 3, 4, 5, 6 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2, 3, 4, 5, 6 | 1.28 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2, 3, 4, 5, 6 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| E-UTRAN PRACH configuration index |  |  | 1, 2, 3 | 534 | As specified in table 5.7.1-2 in TS 36.211 [23] |
|  |  |  | 4, 5, 6 |  |  |
| T1 |  | s | 1, 2, 3, 4, 5, 6 | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3, 4, 5, 6 | 75 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.6.1.2.2.2-3: Cell specific test parameters for NR cell 1

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1, 4 | N/A |  |
|  |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1, 4 | SR.1.1 FDD |  |
|  |  | 2, 5 | SR.1.1 TDD |  |
|  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORESET RMC |  | 1, 4 | CR.1.1 FDD |  |
| configuration |  | 2, 5 | CR.1.1 TDD |  |
|  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORESET RMC |  | 1, 4 | CCR.1.1 FDD |  |
| configuration |  | 2, 5 | CCR.1.1 TDD |  |
|  |  | 3, 6 | CCR.2.1 TDD |  |
| SSB configuration |  | 1, 4 | SSB.1 FR1 |  |
|  |  | 2, 5 | SSB.1 FR1 |  |
|  |  | 3, 6 | SSB.2 FR1 |  |
| SMTC configuration |  | 1, 4 | SMTC.2 |  |
|  |  | 2, 5 | SMTC.1 |  |
|  |  | 3, 6 | SMTC.1 |  |
| OCNG Pattern |  | 1, 2, 3, 4, 5, 6 | OP.1 defined in A.3.2.1 |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2, 3, 4, 5, 6 | SSB |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4, 5 | -140 |  |
|  |  | 3, 6 | -137 |  |
|  | dBm/SCS | 1, 4 | -98 |  |
|  |  | 2, 5 | -98 |  |
|  |  | 3, 6 | -95 |  |
|  | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| SS-RSRP | dBm/SCS | 1, 4 | -102 | -86 |
|  |  | 2, 5 | -102 | -86 |
|  |  | 3, 6 | -99 | -83 |
|  | dB | 1, 4 | -4 | 12 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
|  | dB | 1, 4 | -4 | 12 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
| Io | dBm/9.36 MHz | 1, 4 | -68.60 | -57.78 |
|  | dBm/9.36 MHz | 2, 5 | -68.60 | -57.78 |
|  | dBm/38.16 MHz | 3, 6 | -62.50 | -51.69 |
| Treselection | S | 1, 2, 3, 4, 5, 6 | 0 |  |
| SnonintrasearchP | dB | 1, 2, 3, 4, 5, 6 | Not sent |  |
| Threshx, highP | dB | 1, 2, 3, 4, 5, 6 | 48 |  |
| Threshserving, lowP | dB | 1, 2, 3, 4, 5, 6 | 44 |  |
| Threshx, lowP (Note 2) | dB | 1, 2, 3, 4, 5, 6 | 50 |  |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, high which is included in NR system information, and is a threshold for the E-UTRA target cell |  |  |  |  |

Table A.6.1.2.2.2-4: Cell specific test parameters for E-UTRA cell 2

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm | -140 |  |
|  | dBm/15 kHz | -98 |  |
| RSRP | dBm/15 KHz | -84 | -84 |
|  | dB | 14 | 14 |
|  | dB | 14 | 14 |
| TreselectionEUTRAN | S | 0 |  |
| SnonintrasearchP | dB | Not sent |  |
| Threshx, highP (Note 2) | dB | 48 |  |
| Threshserving, lowP | dB | 44 |  |
| Threshx, lowP | dB | 50 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, high which is included in E-UTRA system information, and is a threshold for the NR target cell |  |  |  |

##### A.6.1.2.2.3	Test Requirements

The cell reselection delay to a lower priority E-UTRAN cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to a lower priority cell shall be less than 8 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a lower priority cell can be expressed as: Tevaluate, E-UTRAN + TSI-E-UTRA,

Where:

Tevaluate, E-UTRAN	See Table 4.2.2.5-1 in clause 4.2.2.5

TSI-E-UTRA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 7.68 s, allow 8 s for the cell re-selection delay to a lower priority E-UTRAN cell.

#### A.6.1.2.3	Cell reselection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion

##### A.6.1.2.3.1	Test Purpose and Environment

This test is to verify the requirement for the NR to E-UTRAN inter-RAT cell reselection when UE fulfills the low mobility criterion specified in clause 4.2.2.11.2 and the E-UTRAN cell is of lower priority.

##### A.6.1.2.3.2	Test Parameters

The test scenario comprises of one NR cell and one E-UTRAN cell as given in tables A.6.1.2.3.2-1, A.6.1.2.3.2-2, A.6.1.2.3.2-3 and A.6.1.2.3.2-4. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both NR cell 1 and E-UTRAN cell 2 are already identified by the UE prior to the start of the test. E-UTRAN cell 2 is of lower priority than cell 1.

As specified in the Test Purpose, the UE is configured with the relaxed measurement criterion for UE with low mobility defined in clause 5.2.4.9.1 in [1]. So, Cell 1 configures the UE as follows:

-	lowMobilityEvalutation [2] criterion is configured according to the parameters listed in Table A.6.1.2.3.2-3;

-	cellEdgeEvaluation [2] criterion is not configured;

-	combineRelaxedMeasCondition [2] is not configured

Table A.6.1.2.3.2-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.6.1.2.3.2-2: General test parameters for NR to E-UTRAN cell re-selection test case for UE fulfilling low mobility criterion

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE camps on cell 1 in the initial phase, it fulfills Low Mobility relaxation measurements criterion, and during T1 period the UE reselects to cell 2 |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6 | Cell2 |  |
| T1 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell2 | The UE shall perform reselection to cell 2 during T1 |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6 | Cell1 |  |
| T2 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE shall perform reselection to cell 1 with higher priority during T2 for iteration of the tests. |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6 | Cell2 |  |
| Access Barring Information |  | - | 1, 2, 3, 4, 5, 6 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2, 3, 4, 5, 6 | 0.64 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2, 3, 4, 5, 6 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| E-UTRAN PRACH configuration index |  |  | 1, 2, 3 | 53 | As specified in table 5.7.1-2 in TS 36.211 [23] |
|  |  |  | 4, 5, 6 | 4 |  |
| T1 |  | s | 1, 2, 3, 4, 5, 6 | 24 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3, 4, 5, 6 | 24 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.6.1.2.3.2-3: Cell specific test parameters for NR cell 1

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1, 4 | N/A |  |
|  |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1, 4 | SR.1.1 FDD |  |
|  |  | 2, 5 | SR.1.1 TDD |  |
|  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORESET RMC |  | 1, 4 | CR.1.1 FDD |  |
| configuration |  | 2, 5 | CR.1.1 TDD |  |
|  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORESET RMC |  | 1, 4 | CCR.1.1 FDD |  |
| configuration |  | 2, 5 | CCR.1.1 TDD |  |
|  |  | 3, 6 | CCR.2.1 TDD |  |
| SSB configuration |  | 1, 4 | SSB.1 FR1 |  |
|  |  | 2, 5 | SSB.1 FR1 |  |
|  |  | 3, 6 | SSB.2 FR1 |  |
| SMTC configuration |  | 1, 4 | SMTC pattern 2 |  |
|  |  | 2, 5 | SMTC pattern 1 |  |
|  |  | 3, 6 | SMTC pattern 1 |  |
| OCNG Pattern |  | 1, 2, 3, 4, 5, 6 | OP.1 defined in A.3.2.1 |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2, 3, 4, 5, 6 | SSB |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4, 5 | -140 |  |
|  |  | 3, 6 | -137 |  |
|  | dBm/SCS | 1, 4 | -98 |  |
|  |  | 2, 5 | -98 |  |
|  |  | 3, 6 | -95 |  |
|  | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| SS-RSRP | dBm/SCS | 1, 4 | -102 | -86 |
|  |  | 2, 5 | -102 | -86 |
|  |  | 3, 6 | -99 | -83 |
|  | dB | 1, 4 | -4 | 12 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
|  | dB | 1, 4 | -4 | 12 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
| Io | dBm/9.36 MHz | 1, 4 | -68.60 | -57.78 |
|  | dBm/9.36 MHz | 2, 5 | -68.60 | -57.78 |
|  | dBm/38.16 MHz | 3, 6 | -62.50 | -51.69 |
| TreselectionP | S | 1, 2, 3, 4, 5, 6 | 0 |  |
| Snonintrasearch | dB | 1, 2, 3, 4, 5, 6 | 50 |  |
| Threshx, highP | dB | 1, 2, 3, 4, 5, 6 | 48 |  |
| Threshserving, lowP | dB | 1, 2, 3, 4, 5, 6 | 44 |  |
| Threshx, lowP (Note 2) | dB | 1, 2, 3, 4, 5, 6 | 50 |  |
| SSearchDeltaP | dB | 1, 2, 3, 4, 5, 6 | 3 |  |
| TSearchDeltaP | s | 1, 2, 3, 4, 5, 6 | 5 |  |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, low which is included in NR system information, and is a threshold for the E-UTRA target cell |  |  |  |  |

Table A.6.1.2.3.2-4: Cell specific test parameters for E-UTRA cell 2

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm | -140 |  |
|  | dBm/15 kHz | -98 |  |
| RSRP | dBm/15 KHz | -84 | -84 |
|  | dB | 14 | 14 |
|  | dB | 14 | 14 |
| TreselectionEUTRAN | S | 0 |  |
| Snonintrasearch | dB | Not sent |  |
| Threshx, high (Note 2) | dB | 48 |  |
| Threshserving, low | dB | 44 |  |
| Threshx, low | dB | 50 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, high which is included in E-UTRA system information, and is a threshold for the NR target cell |  |  |  |

##### A.6.1.2.3.3	Test Requirements

The cell reselection delay to a lower priority E-UTRAN cell with UE fulfilling low mobility criterion is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCConnectionRequest message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to a lower priority cell shall be less than 17  s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a lower priority cell can be expressed as: Tevaluate, E-UTRAN + TSI-E-UTRA,

Where:

Tevaluate, E-UTRAN	See Table 4.2.2.11.2-1 in clause 4.2.2.11.2

TSI-E-UTRA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 15.36 (Tevaluate, E-UTRAN) + 1.28 (TSI-E-UTRA) = 16.64 s, allow 17 s for the cell re-selection delay to a lower priority E-UTRAN cell for UE fulfilling low mobility criterion.

#### A.6.1.2.4	Cell reselection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion

##### A.6.1.2.4.1	Test Purpose and Environment

This test is to verify the requirement for the NR to E-UTRAN inter-RAT cell reselection requirements when UE fulfills  not-at-cell edge criterion specified in clause 4.2.2.11.3 when the E-UTRAN cell is of lower priority.

##### A.6.1.2.4.2	Test Parameters

The test scenario comprises of one NR cell and one E-UTRAN cell as given in tables A.6.1.2.4.2-1, A.6.1.2.4.2-2, A.6.1.2.4.2-3 and A.6.1.2.4.2-4. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both NR cell 1 and E-UTRAN cell 2 are already identified by the UE prior to the start of the test. E-UTRAN cell 2 is of lower priority than cell 1.

As specified in the Test Purpose, the UE is configured with the relaxed measurement criterion for UE with not-at-cell edge defined in clause 5.2.4.9.2 in [1]. So, Cell 1 configures the UE as follows:

-	lowMobilityEvalutation [2] criterion is not configured;

-	cellEdgeEvaluation [2] criterion is configured according to the parameters listed in Table A.6.1.2.4.2-3;

-	combineRelaxedMeasCondition [2] is not configured

Table A.6.1.2.4.2-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.6.1.2.4.2-2: General test parameters for NR to E-UTRAN cell re-selection test case for UE fulfilling not-at-cell edge criterion

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE camps on cell 1 in the initial phase and fulfill the not at the cell edge criteria. |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6 | Cell2 |  |
| T1 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell2 | The UE shall perform reselection to cell 2 during T1. |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6 | Cell1 |  |
| T2 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE shall perform reselection to cell 1 during T2 for iteration of the tests. |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6 | Cell2 |  |
| Access Barring Information |  | - | 1, 2, 3, 4, 5, 6 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2, 3, 4, 5, 6 | 0.64 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2, 3, 4, 5, 6 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| E-UTRAN PRACH configuration index |  |  | 1, 2, 3 | 53 | As specified in table 5.7.1-2 in TS 36.211 [23] |
|  |  |  | 4, 5, 6 | 4 |  |
| T1 |  | s | 1, 2, 3, 4, 5, 6 | 24 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3, 4, 5, 6 | 24 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.6.1.2.4.2-3: Cell specific test parameters for NR cell 1

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1, 4 | N/A |  |
|  |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1, 4 | SR.1.1 FDD |  |
|  |  | 2, 5 | SR.1.1 TDD |  |
|  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORESET RMC |  | 1, 4 | CR.1.1 FDD |  |
| configuration |  | 2, 5 | CR.1.1 TDD |  |
|  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORESET RMC |  | 1, 4 | CCR.1.1 FDD |  |
| configuration |  | 2, 5 | CCR.1.1 TDD |  |
|  |  | 3, 6 | CCR.2.1 TDD |  |
| SSB configuration |  | 1, 4 | SSB.1 FR1 |  |
|  |  | 2, 5 | SSB.1 FR1 |  |
|  |  | 3, 6 | SSB.2 FR1 |  |
| SMTC configuration |  | 1, 4 | SMTC pattern 2 |  |
|  |  | 2, 5 | SMTC pattern 1 |  |
|  |  | 3, 6 | SMTC pattern 1 |  |
| OCNG Pattern |  | 1, 2, 3, 4, 5, 6 | OP.1 defined in A.3.2.1 |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2, 3, 4, 5, 6 | SSB |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4, 5 | -140 |  |
|  |  | 3, 6 | -137 |  |
|  | dBm/SCS | 1, 4 | -98 |  |
|  |  | 2, 5 | -98 |  |
|  |  | 3, 6 | -95 |  |
|  | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| SS-RSRP | dBm/SCS | 1, 4 | -102 | -86 |
|  |  | 2, 5 | -102 | -86 |
|  |  | 3, 6 | -99 | -83 |
|  | dB | 1, 4 | -4 | 12 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
|  | dB | 1, 4 | -4 | 12 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
| SSearchThresholdP | dB | 1, 2, 3, 4, 5, 6 | 32 | 32 |
| Io | dBm/9.36 MHz | 1, 4 | -68.60 | -57.78 |
|  | dBm/9.36 MHz | 2, 5 | -68.60 | -57.78 |
|  | dBm/38.16 MHz | 3, 6 | -62.50 | -51.69 |
| Treselection | S | 1, 2, 3, 4, 5, 6 | 0 |  |
| SnonintrasearchP | dB | 1, 2, 3, 4, 5, 6 | 60 |  |
| Threshx, highP | dB | 1, 2, 3, 4, 5, 6 | 48 |  |
| Threshserving, lowP | dB | 1, 2, 3, 4, 5, 6 | 44 |  |
| Threshx, lowP (Note 2) | dB | 1, 2, 3, 4, 5, 6 | 50 |  |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, low which is included in NR system information, and is a threshold for the E-UTRA target cell |  |  |  |  |

Table A.6.1.2.4.2-4: Cell specific test parameters for E-UTRA cell 2

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm | -140 |  |
|  | dBm/15 kHz | -98 |  |
| RSRP | dBm/15 KHz | -84 | -84 |
|  | dB | 14 | 14 |
|  | dB | 14 | 14 |
| TreselectionEUTRAN | S | 0 |  |
| Snonintrasearch | dB | Not sent |  |
| Threshx, high (Note 2) | dB | 48 |  |
| Threshserving, low | dB | 44 |  |
| Threshx, low | dB | 50 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, high which is included in E-UTRA system information, and is a threshold for the NR target cell |  |  |  |

##### A.6.1.2.4.3	Test Requirements

The cell reselection delay to a lower priority E-UTRAN cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCConnectionRequest message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to a lower priority cell shall be less than 17s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a lower priority cell can be expressed as: Tevaluate, E-UTRAN + TSI-E-UTRA,

Where:

Tevaluate, E-UTRAN	See Table 4.2.2.11.3-1 in clause 4.2.2.11.3

TSI-E-UTRA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 16.64 s, allow 17 s for the cell re-selection delay to a lower priority E-UTRAN cell for UE fulfilling not-at-cell edge criterion.

#### A.6.1.2.5	Cell reselection to lower priority E-UTRAN cell for UE configured with highSpeedMeasFlag-r16

##### A.6.1.2.5.1	Test Purpose and Environment

This test is to verify the requirement for the NR to E-UTRAN inter-RAT cell reselection requirements for UE configured with highSpeedMeasFlag-r16 specified in clause 4.2.2.5 when the E-UTRAN cell is of lower priority.

##### A.6.1.2.5.2	Test Parameters

The test scenario comprises of one NR cell and one E-UTRAN cell as given in tables A.6.1.2.5.2-1, A.6.1.2.5.2-2, A.6.1.2.5.2-3 and A.6.1.2.5.2-4. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both NR cell 1 and E-UTRAN cell 2 are already identified by the UE prior to the start of the test. E-UTRAN cell 2 is of lower priority than cell 1. The E-UTRAN cell 2 is indicated by NR cell 1 as an HST cell.

Table A.6.1.2.5.2-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.6.1.2.5.2-2: General test parameters for NR to E-UTRAN cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE camps on cell 1 in the initial phase. |
| T1 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell2 | The UE shall perform reselection to cell 2 during T1. |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6 | Cell1 |  |
| T2 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE shall perform reselection to cell 1 during T2 for iteration of the tests. |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6 | Cell2 |  |
| Access Barring Information |  | - | 1, 2, 3, 4, 5, 6 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2, 3, 4, 5, 6 | 0.32 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2, 3, 4, 5, 6 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| E-UTRAN PRACH configuration index |  |  | 1, 2, 3 | 53 | As specified in table 5.7.1-2 in TS 36.211 [23] |
|  |  |  | 4, 5, 6 | 4 |  |
| T1 |  | s | 1, 2, 3, 4, 5, 6 | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3, 4, 5, 6 | 75 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.6.1.2.5.2-3: Cell specific test parameters for NR cell 1

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1, 4 | N/A |  |
|  |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1, 4 | SR.1.1 FDD |  |
|  |  | 2, 5 | SR.1.1 TDD |  |
|  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1, 4 | CR.1.1 FDD |  |
|  |  | 2, 5 | CR.1.1 TDD |  |
|  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1, 4 | CCR.1.1 FDD |  |
|  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  | 3, 6 | CCR.2.1 TDD |  |
| SSB configuration |  | 1, 4 | SSB.1 FR1 |  |
|  |  | 2, 5 | SSB.1 FR1 |  |
|  |  | 3, 6 | SSB.2 FR1 |  |
| SMTC configuration |  | 1, 4 | SMTC pattern 2 |  |
|  |  | 2, 5 | SMTC pattern 1 |  |
|  |  | 3, 6 | SMTC pattern 1 |  |
| OCNG Pattern |  | 1, 2, 3, 4, 5, 6 | OP.1 defined in A.3.2.1 |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2, 3, 4, 5, 6 | SSB |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4, 5 | -140 |  |
|  |  | 3, 6 | -137 |  |
|  | dBm/SCS | 1, 4 | -98 |  |
|  |  | 2, 5 | -98 |  |
|  |  | 3, 6 | -95 |  |
|  | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| SS-RSRP | dBm/SCS | 1, 4 | -102 | -86 |
|  |  | 2, 5 | -102 | -86 |
|  |  | 3, 6 | -99 | -83 |
|  | dB | 1, 4 | -4 | 12 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
|  | dB | 1, 4 | -4 | 12 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
| Io | dBm/9.36 MHz | 1, 4 | -68.60 | -57.78 |
|  | dBm/9.36 MHz | 2, 5 | -68.60 | -57.78 |
|  | dBm/38.16 MHz | 3, 6 | -62.50 | -51.69 |
| Treselection | S | 1, 2, 3, 4, 5, 6 | 0 |  |
| SnonintrasearchP | dB | 1, 2, 3, 4, 5, 6 | 50 |  |
| Threshx, highP (Note 2) | dB | 1, 2, 3, 4, 5, 6 | 48 |  |
| Threshserving, lowP | dB | 1, 2, 3, 4, 5, 6 | 44 |  |
| Threshx, lowP | dB | 1, 2, 3, 4, 5, 6 | 50 |  |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN 1944HzNote3 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, highP which is included in NR system information, and is a threshold for the E-UTRA target cell. Note 3: The AWGN 1944 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 1944 Hz. |  |  |  |  |

Table A.6.1.2.5.2-4: Cell specific test parameters for E-UTRA cell 2

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm | -140 |  |
|  | dBm/15 kHz | -98 |  |
| RSRP | dBm/15 KHz | -84 | -84 |
|  | dB | 14 | 14 |
|  | dB | 14 | 14 |
| TreselectionEUTRAN | S | 0 |  |
| SnonintrasearchP | dB | Not sent |  |
| Threshx, highP (Note 2) | dB | 48 |  |
| Threshserving, lowP | dB | 44 |  |
| Threshx, lowP | dB | 50 |  |
| Propagation Condition |  | AWGN 1944Hz |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, highP which is included in E-UTRA system information, and is a threshold for the NR target cell |  |  |  |

##### A.6.1.2.5.3	Test Requirements

The cell reselection delay to a lower priority E-UTRAN cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to a lower priority cell shall be less than 3 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a lower priority cell can be expressed as: Tevaluate, E-UTRAN_HST + TSI-E-UTRA,

Where:

Tevaluate, E-UTRAN_HST	See Table 4.2.2.5-2 in clause 4.2.2.5

TSI-E-UTRA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 2.24 s, allow 3 s for the cell re-selection delay to a lower priority E-UTRAN cell.

#### A.6.1.1.7	Void

## A.6.2	SA: RRC_INACTIVE state mobility

### A.6.2.1	Configured Grant based Small Data Transmissions (CG-SDT)

#### A.6.2.1.1	Test purpose and Environment

The purpose of this test is to verify that the UE properly perform TA validation for CG-SDT transmission in clause 5.5.3. In the test, UE is configured two CG-SDT configurations at different time. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 repectively. There is one cell, which is the active NR cell in FR1.  Figure A.6.2.1.1-1 shows the variation of the RSRP per time. Prior to the time point TA, the UE shall be fully synchronized to PCell (Cell 1).

Before starting the test at time point TA, test equipment configures RSRP to P0.

At time point TB, RSRP is changed from P0 to P1.

At time point TC, UE expect to receive RRC release with CG SDT configuration and RRC status is changed to INACTIVE status.

At time point TD, RSRP is changed from P1 to P0.

At time point TE, RSRP is changed from P0 to P2.

Test equipment triggers UL data arrival at UE lower layer at time point TF.

At time point TH, UE expect to receive a RRC release with CG-SDT configurations.

At time point TI, RSRP is changed from P2 to P3.

Test equipment triggers UL data arrival at UE lower layer at time point TJ.

Figure A.6.2.1.1-1: RSRP variation model for CG-SDT testing

#### A.6.2.1.2	Test Parameters

There is one cells in the test, the FR1 PCell. The test parameters for the PCell are given in Table A.6.2.1.2-1, Table A.6.2.1.2-2, and Table A.6.2.1.2-3.

Table A.6.2.1.2-1: NR configuration for FR1 SSB

| Config | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.2.1.2-2 : General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| TDD Configuration |  | 1 | N/A |  |
|  |  | 2 | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  |
| BWchannel | MHz |  | 10: NRB,c = 52 |  |
| PDSCH Reference measurement channel |  | 1 | SR.1.1 FDD |  |
|  |  | 2 | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | 1 | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | 1 | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  |
| SSB configuration |  | 1,2,3 | SSB.1 FR1 |  |
| OCNG Patterns |  | 1,2,3 | OP.1 |  |
| Initial BWP Configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration |  | 1,2,3 | DLBWP.1.1 ULBWP.1.1 |  |
| SMTC configuration |  | 1,2,3 | SMTC.1 |  |
| DRX configuration |  | 1,2,3 | DRX.7 |  |
| T1 | s | 1,2,3 | [1] |  |
| T2 | s | 1,2,3 | [1.28] |  |
| T3 | s | 1,2,3 | [2.36] |  |
| T4 | s | 1,2,3 | [2.56] |  |
| T5 | s | 1,2,3 | [2.36] |  |
| cg-SDT-RSRP-ChangeThreshold | dB | 1,2,3 | [8] |  |
| cg-SDT-TimeAlignmentTime |  | 1,2,3 | infinity |  |
| CG-SDT resource period | ms |  | 640 | configuredGrantConfig Peridocity at 38.331 Config 1,2 : sym640x14, config 3 : sym1024x14 |
| Propagation condition |  | 1,2,3 | AWGN |  |

Table A.6.2.1.2-3: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
|  | 1,2,3 | dBm/15kHz | -100 |  |  |  |  |
|  | 1,2 | dBm/SSB SCS | -100 |  |  |  |  |
|  | 3 | dBm/SSB SCS | -103 |  |  |  |  |
|  | 1,2,3 | dB | [0] | [10.35] | [0] | [15.25] | [6] |
| SS RSRP | 1,2,3 | dBm/SSB SCS | [-100] | [-89.65] | [-100] | [-84.75] | [-96] |
| Io | 1,2 | dBm/9.36MHz | [-69.04] | [-61.31] | [-69.04] | [-56.67] | [-65.07] |
|  | 3 | dBm/38.16MHz | [-69.04] | [-61.31] | [-69.04] | [-56.67] | [-65.07] |
|  | 1,2,3 | dB | [0] | [10.35] | [0] | [15.25] | [6] |

#### A.6.2.1.3	Test requirements

The UE behaviour in each test during time durations shall be as follows:

During T4, UE shall transmit PUSCH at CG-SDT resource within 640ms + Z after time point TF.

During T5, UE shall not transmit PUSCH at CG-SDT resources after TJ until the end of the test at time point TK.

The rate of correct events observed during repeated tests shall be at least 90%.

## A.6.3	RRC_CONNECTED state mobility

### A.6.3.1	Handover

#### A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell

##### A.6.3.1.1.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 intra frequency handover requirements specified in clause 6.1.1.2.

##### A.6.3.1.1.2	Test Parameters

Supported test configurations are shown in table A.6.3.1.1.2-1. Both handover delay and interruption length are tested by using the parameters in table A.6.3.1.1.2-2, and A.6.3.1.1.2-3.

The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2.

NR shall send a RRC message implying handover to cell 2. The RRC message implying handover shall be sent to the UE during period T2, after the UE has reported Event A3. T3 is defined as the end of the last TTI containing the RRC message implying handover.

Table A.6.3.1.1.2-1: Intra-frequency handover from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.1.1.2-2: General test parameters Intra-frequency handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

Table A.6.3.1.1.2-3: Cell specific test parameters for NR FR1-FR1 Intra frequency handover test case

| Parameter |  |  | Unit | Cell 1 |  |  |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T3 |  | T1 |  | T2 |  | T3 |
| NR RF Channel Number |  |  |  | 1 |  |  |  |  |  | 1 |  |  |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |  |
| PDSCH Reference |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |  |  |  |  |  |
| measurement channel |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |  |  |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |  |  |  |  |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | dB | 8 | -3.3 |  | -3.3 |  | -Infinity |  | 2.36 |  | 2.36 |  |
|  |  |  | dB | 8 | 8 |  | 8 |  | -Infinity |  | 11 |  | 11 |  |
| SSB_RP | Config 1,2 |  | dBm/SCS | -90 | -90 |  | -90 |  | -Infinity |  | -87 |  | -87 |  |
|  | Config 3 |  | dBm/SCS | -87 | -87 |  | -87 |  | -Infinity |  | -84 |  | -84 |  |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -61.41 | -57.06 |  | -57.06 |  | -61.41 |  | -57.06 |  | -57.06 |  |
|  | Config 3 |  | dBm/ 38.16MHz | -55.31 | -50.96 |  | -50.96 |  | -55.31 |  | -50.96 |  | -50.96 |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.3.1.1.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 72 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt = 62 ms in the test. Tinterrupt is defined in clause 6.1.1.2.2.

This gives a total of 72 ms.

#### A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell

##### A.6.3.1.2.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 intra frequency handover requirements specified in clause 6.1.1.2.

##### A.6.3.1.2.2	Test Parameters

Supported test configurations are shown in table A.6.3.1.2.2-1. Both handover delay and interruption length are tested by using the parameters in table A.6.3.1.2.2-2, and A.6.3.1.2.2-3.

The test scenario comprises of two carriers and one cell on each carrier. No gap patterns are configured in the test case. The test consists of two successive time periods, with time durations of T1, T2 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE receives a RRC handover command from the network. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE.

Table A.6.3.1.2.2-1: Intra-frequency handover from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.1.2.2-2: General test parameters Intra-frequency handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |

Table A.6.3.1.2.2-3: Cell specific test parameters for NR FR1-FR1 Intra frequency handover test case

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 1 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf. 2.1 |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |  |  |  |
|  |  |  | dB | 8 | -0.64 |  | -Infinity |  | -0.64 |  |
|  |  |  | dB | 8 | 8 |  | -Infinity |  | 8 |  |
| SSB_RP | Config 1,2 |  | dBm/SCS | -90 | -90 |  | -Infinity |  | -90 |  |
|  | Config 3 |  | dBm/SCS | -87 | -87 |  | -Infinity |  | -87 |  |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -61.41 | -58.71 |  | -61.41 |  | -58.71 |  |
|  | Config 3 |  | dBm/ 38.16MHz | -55.31 | -52.60 |  | -55.31 |  | -52.60 |  |
| Propagation condition |  |  | - | AWGN |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

##### A.6.3.1.2.3	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 92 ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt = 82 ms in the test. Tinterrupt is defined in clause 6.1.1.2.2.

This gives a total of 92 ms.

#### A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell

##### A.6.3.1.3.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 inter frequency handover requirements specified in clause 6.1.1.2.

##### A.6.3.1.3.2	Test Parameters

Supported test configurations are shown in table A.6.3.1.3.2-1. Both handover delay and interruption length are tested by using the parameters in table A.6.3.1.3.2-2, and A.6.3.1.3.2-3.

The test scenario comprises of two carriers and one cell on each carrier. No gap patterns are configured in the test case. The test consists of two successive time periods, with time durations of T1, T2 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE receives a RRC handover command from the network. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE.

Table A.6.3.1.3.2-1: Inter-frequency handover from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.1.3.2-2: General test parameters Inter-frequency handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |

Table A.6.3.1.3.2-3: Cell specific test parameters for NR FR1-FR1 Inter frequency handover test case

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 2 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |
| BWP |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  | -98 |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  | -98 |  |  |  |
|  | Config 3 |  |  | -95 |  |  | -95 |  |  |  |
|  |  |  | dB | 4 | 4 |  | -Infinity |  | 5 |  |
|  |  |  | dB | 4 | 4 |  | -Infinity |  | 5 |  |
| SSB_RP | Config 1,2 |  | dBm/SCS | -94 | -94 |  | -Infinity |  | -93 |  |
|  | Config 3 |  | dBm/SCS | -91 | -91 |  | -Infinity |  | -90 |  |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -64.59 | -64.59 |  | -70.05 |  | -63.85 |  |
|  | Config 3 |  | dBm/ 38.16MHz | -58.49 | -58.49 |  | -63.94 |  | -57.75 |  |
| Propagation condition |  |  | - | AWGN |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

##### A.6.3.1.3.3	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 132 ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt = 122 ms in the test. Tinterrupt is defined in clause 6.1.1.2.2.

This gives a total of 132 ms.

#### A.6.3.1.4	SA NR - E-UTRAN handover

##### A.6.3.1.4.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE can make correct inter-RAT E-UTRAN handover when operating in standalone (SA) operation with PCell in FR1. This test shall verify the NR to E-UTRAN handover requirements as specified in clause 6.1.2.1.

The test comprises of one NR carrier and one E-UTRA carrier. There are two cells and one cell on each carrier. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN neighbour cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of Cell 2. Starting T2, Cell 2 becomes detectable and the UE is expected to detect and send a measurement report. Gap pattern configuration with id #0 as specified in Table 9.1.2-1 is configured before T2 begins to enable inter-RAT frequency monitoring.

A RRC message implying handover shall be sent to the UE during period T2 after the UE has reported Event B2. The start of T3 is the instant when the last TTI containing the RRC message implying handover is sent to the UE. The handover message shall contain Cell 2 as the target cell.

Supported test configurations are shown in table A.6.3.1.4-1. General test parameters are provided in Table A.6.3.1.4-2. Cell specific test parameters for Cell 1 and Cell 2 are provided in Tables A.6.3.1.4-3 and A.6.3.1.4-4 respectively.

Table A.6.3.1.4-1: Supported test configurations for SA inter-RAT E-UTRAN handover tests

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.1.4-2: General test parameters for SA inter-RAT E-UTRAN handover

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  |  | 2 | 1 E-UTRAN carrier frequency is used in the test |
| Initial conditions | Active cell |  | Cell 1 | NR cell |
|  | Neighbouring cell |  | Cell 2 | E-UTRAN cell |
| Final condition | Active cell |  | Cell 2 |  |
| NR measurement quantity |  |  | SS-RSRP |  |
| E-UTRAN measurement quantity |  |  | RSRP |  |
| b2-Threshold1 |  | dBm | As specified in Table A.6.3.1.4-3 | Absolute NR SS-RSRP threshold for event B2 |
| b2-Threshold2EUTRAN |  | dBm | -98 | Absolute E-UTRAN RSRP threshold for event B2 |
| Hysteresis |  | dB | 0 |  |
| TimeToTrigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  | OFF | Non-DRX test |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| Gap pattern configuration Id |  |  | 0 | As specified in Table 9.1.2-1 started before T2 starts |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

Table A.6.3.1.4-3: Cell specific test parameters for SA inter-RAT E-UTRA handover (Cell 1)

| Parameter |  | Unit | Configuration | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 |
| RF channel number |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |  |
| Duplex mode |  |  | 1, 4 | FDD |  |  |
|  |  |  | 2, 3, 5, 6 | TDD |  |  |
| TDD Configuration |  |  | 2, 5 | TDDConf.1.1 |  |  |
|  |  |  | 3, 6 | TDDConf.2.1 |  |  |
| BWchannel |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |  |
|  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |  |
|  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |  |
| PDSCH reference measurement channel |  |  | 1, 4 | SR.1.1 FDD |  |  |
|  |  |  | 2, 5 | SR.1.1 TDD |  |  |
|  |  |  | 3, 6 | SR.2.1 TDD |  |  |
| CORSET reference channel |  |  | 1, 4 | CR.1.1 FDD |  |  |
|  |  |  | 2, 5 | CR.1.1 TDD |  |  |
|  |  |  | 3, 6 | CR.2.1 TDD |  |  |
| TRS configuration |  |  | 1, 4 | TRS.1.1 FDD |  |  |
|  |  |  | 2, 5 | TRS.1.1 TDD |  |  |
|  |  |  | 3, 6 | TRS.1.2 TDD |  |  |
| OCNG patternNote1 |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |  |
| BWP | Initial DL BWP |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |
| SMTC configuration |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |  |
| SSB configuration |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |  |
|  |  |  | 3, 6 | SSB.2 FR1 |  |  |
| b2-Threshold1 |  | dBm | 1, 2, 4, 5 | -96 |  |  |
|  |  |  | 3, 6 | -93 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -100 | -104 | -100 |
| NocNote2 |  | dBm/SCS | 1, 2, 4, 5 | -100 | -104 | -100 |
|  |  |  | 3, 6 | -97 | -101 | -97 |
| Ês/Noc |  | dB | 1, 2, 3, 4, 5, 6 | 12 | 0 | -4 |
| Ês/IotNote3 |  | dB | 1, 2, 3, 4, 5, 6 | 12 | 0 | -4 |
| SS-RSRPNote3 |  | dBm/SCS | 1, 2, 4, 5 | -88 | -104 | -104 |
|  |  |  | 3, 6 | -85 | -101 | -101 |
| IoNote3 |  | dBm/9.36 MHz | 1, 2, 4, 5 | -59.78 | -73.04 | -70.59 |
|  |  | dBm/38.16 MHz | 3, 6 | -53.68 | -66.9448 | -64.49 |
| Propagation condition |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.3.1.4-4: Cell specific test parameters for SA inter-RAT E-UTRA handover (Cell 2)

| Parameter | Unit | Configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |  |
|  |  | 4, 5, 6 | TDD |  |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |  |
| PRACH ConfigurationNote2 |  | 1, 2, 3 | 4 |  |  |
|  |  | 4, 5, 6 | 53 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote3 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote3 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |  |
| OCNG PatternsNote3 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote4 |  |  |  |  |  |
| OCNG_RBNote4 |  |  |  |  |  |
| NocNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -98 |  |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 8 | 78 |
| Ês/IotNote6 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 78 | 78 |
| RSRPNote6 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -90 | -90 |
| SCH_RPNote6 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -90 | -90 |
| IoNote6 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -67.21 +10log(NRB,c/100) | -58.57 +10log(NRB,c/100) | -58.57 +10log(NRB,c/100) |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix Note7 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: PRACH configurations are specified in table 5.7.1-2 and table 5.7.1-3 in TS 36.211 [23]. Note 3: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 4: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 6: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 7: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |  |

##### A.6.3.1.4.2	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 85 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 50 ms and is specified in clause 6.1.2.1.

Tinterrupt = 35 ms in the test; Tinterrupt is defined in clause 6.1.2.1.

This gives a total of 85 ms.

#### A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell

##### A.6.3.1.5.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE can make correct inter-RAT E-UTRAN handover when operating in standalone (SA) operation with PCell in FR1. This test shall verify the NR to E-UTRAN handover requirements for the case when the target E-UTRAN cell is unknown as specified in clause 6.1.2.1.

The test comprises of one NR carrier and one E-UTRA carrier. There are two cells and one cell on each carrier. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN neighbour cell. The test consists of two successive time periods, with time durations of T1 and T2 respectively. At the start of time duration T1, the UE does not have any timing information of Cell 2. Starting T2, Cell 2 becomes detectable. No Gap pattern shall be configured.

A RRC message implying handover shall be sent to the UE during period T1. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE. The handover message shall contain Cell 2 as the target cell.

Supported test configurations are shown in table A.6.3.1.5-1. General test parameters are provided in Table A.6.3.1.5-2. Cell specific test parameters for Cell 1 and Cell 2 are provided in Tables A.6.3.1.5-3 and A.6.3.1.5-4 respectively.

Table A.6.3.1.5-1: Supported test configurations for SA inter-RAT E-UTRAN handover tests

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.1.5-2: General test parameters for SA inter-RAT E-UTRAN handover

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  |  | 2 | 1 E-UTRAN carrier frequency is used in the test |
| Initial conditions | Active cell |  | Cell 1 | NR cell |
|  | Neighbouring cell |  | Cell 2 | E-UTRAN cell |
| Final condition | Active cell |  | Cell 2 |  |
| NR measurement quantity |  |  | SS-RSRP |  |
| DRX |  |  | OFF | Non-DRX test |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 1 |  |

Table A.6.3.1.5-3: Cell specific test parameters for SA inter-RAT E-UTRA handover (Cell 1)

| Parameter |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| RF channel number |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  | 1, 4 | FDD |  |
|  |  |  | 2, 3, 5, 6 | TDD |  |
| TDD Configuration |  |  | 2, 5 | TDDConf.1.1 |  |
|  |  |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |
|  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |
|  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  | 3, 6 | SR.2.1 TDD |  |
| CORSET reference channel |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  | 3, 6 | CR.2.1 TDD |  |
| TRS configuration |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  | 3, 6 | TRS.1.2 TDD |  |
| OCNG patternNote1 |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| BWP | Initial DL BWP |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |
| SMTC configuration |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  | 3, 6 | SSB.2 FR1 |  |
| EPRE ratio of PSS to SSS |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| NocNote2 |  | dBm/SCS | 1, 2, 4, 5 | -98 |  |
|  |  |  | 3, 6 | -95 |  |
| Ês/Noc |  | dB | 1, 2, 3, 4, 5, 6 | 0 | 0 |
| Ês/IotNote3 |  | dB | 1, 2, 3, 4, 5, 6 | 0 | 0 |
| SS-RSRPNote3 |  | dBm/SCS | 1, 2, 4, 5 | -98 | -98 |
|  |  |  | 3, 6 | -95 | -95 |
| IoNote3 |  | dBm/9.36 MHz | 1, 2, 4, 5 | -67.04 | -67.04 |
|  |  | dBm/38.16 MHz | 3, 6 | -60.94 | -60.94 |
| Propagation condition |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.6.3.1.5-4: Cell specific test parameters for SA inter-RAT E-UTRA handover (Cell 2)

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PRACH ConfigurationNote2 |  | 1, 2, 3 | 4 |  |
|  |  | 4, 5, 6 | 53 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote3 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote3 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote3 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote4 |  |  |  |  |
| OCNG_RBNote4 |  |  |  |  |
| NocNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 |
| Ês/IotNote6 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 |
| RSRPNote6 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -91 |
| SCH_RPNote6 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -91 |
| IoNote6 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -70.22 | -62.43 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix Note7 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: PRACH configurations are specified in table 5.7.1-2 and table 5.7.1-3 in TS 36.211 [23]. Note 3: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 4: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 6: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 7: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

##### A.6.3.1.5.2	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 165 ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 50 ms and is specified in clause 6.1.2.1.

Tinterrupt = 115 ms in the test; Tinterrupt is defined in clause 6.1.2.1.

This gives a total of 165 ms.

#### A.6.3.1.6	 SA NR - UTRAN FDD handover

##### A.6.3.1.6.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE can make correct inter-RAT UTRAN FDD handover when operating in standalone (SA) operation with PCell in FR1. This test shall verify the NR to UTRAN FDD handover requirements as specified in clause 6.1.2.2.1.

The test comprises of one NR carrier and one UTRA FDD carrier. There are two cells and one cell on each carrier. Cell 1 is the NR PCell and Cell 2 is an inter-RAT UTRAN FDD neighbour cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of Cell 2. Starting T2, Cell 2 becomes detectable and the UE is expected to detect and send a measurement report. Gap pattern configuration with id #0 as specified in Table 9.1.2-1 is configured before T2 begins to enable inter-RAT frequency monitoring.

A RRC message implying handover shall be sent to the UE during period T2 after the UE has reported Event B2. The start of T3 is the instant when the last TTI containing the RRC message implying handover is sent to the UE. The handover message shall contain Cell 2 as the target cell.

Supported test configurations are shown in table A.6.3.1.6-1. General test parameters are provided in Table A.6.3.1.6-2. Cell specific test parameters for Cell 1 and Cell 2 are provided in Tables A.6.3.1.6-3 and A.6.3.1.6-4 respectively.

Table A.6.3.1.6-1: Supported test configurations for SA inter-RAT UTRAN FDD handover tests

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, UTRAN FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, UTRAN FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, UTRAN FDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.1.6-2: General test parameters for SA inter-RAT UTRAN FDD handover

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 NR carrier frequency is used in the test |
| UTRA RF Channel Number |  |  | 2 | 1 UTRAN carrier frequency is used in the test |
| Initial conditions | Active cell |  | Cell 1 | NR cell |
|  | Neighbouring cell |  | Cell 2 | UTRAN cell |
| Final condition | Active cell |  | Cell 2 |  |
| NR measurement quantity |  |  | SS-RSRP |  |
| Inter-RAT (UTRAN FDD) measurement quantity |  |  | CPICH Ec/N0 |  |
| b2-Threshold1 |  | dBm | As specified in Table A.6.3.1.6-3 | Absolute NR SS-RSRP threshold for event B2 |
| b2-Threshold2UTRA-FDD |  | dB | -18 | Absolute UTRAN CPICH Ec/Io threshold for event B2 |
| Hysteresis |  | dB | 0 |  |
| TimeToTrigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  | OFF | Non-DRX test |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| Gap pattern configuration Id |  |  | 0 | As specified in Table 9.1.2-1 started before T2 starts |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

Table A.6.3.1.6-3: Cell specific test parameters for SA inter-RAT UTRAN FDD handover (Cell 1)

| Parameter |  | Unit | Configuration | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 |
| RF channel number |  |  | 1, 2, 3 | 1 |  |  |
| Duplex mode |  |  | 1 | FDD |  |  |
|  |  |  | 2, 3 | TDD |  |  |
| TDD Configuration |  |  | 2 | TDDConf.1.1 |  |  |
|  |  |  | 3 | TDDConf.2.1 |  |  |
| BWchannel |  | MHz | 1 | 10: NRB,c = 52 (FDD) |  |  |
|  |  |  | 2 | 10: NRB,c = 52 (TDD) |  |  |
|  |  |  | 3 | 40: NRB,c = 106 (TDD) |  |  |
| PDSCH reference measurement channel |  |  | 1 | SR.1.1 FDD |  |  |
|  |  |  | 2 | SR.1.1 TDD |  |  |
|  |  |  | 3 | SR.2.1 TDD |  |  |
| CORSET reference channel |  |  | 1 | CR.1.1 FDD |  |  |
|  |  |  | 2 | CR.1.1 TDD |  |  |
|  |  |  | 3 | CR.2.1 TDD |  |  |
| TRS configuration |  |  | 1 | TRS.1.1 FDD |  |  |
|  |  |  | 2 | TRS.1.1 TDD |  |  |
|  |  |  | 3 | TRS.1.2 TDD |  |  |
| OCNG patternNote1 |  |  | 1, 2, 3 | OP.1 |  |  |
| BWP | Initial DL BWP |  | 1, 2, 3 | DLBWP.0.1 |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |
| SMTC configuration |  |  | 1, 2, 3 | SMTC.1 |  |  |
| SSB configuration |  |  | 1, 2 | SSB.1 FR1 |  |  |
|  |  |  | 3 | SSB.2 FR1 |  |  |
| b2-Threshold1 |  | dBm | 1, 2 | -96 |  |  |
|  |  |  | 3 | -93 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 1, 2, 3 | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1, 2, 3 | -100 |  |  |
| NocNote2 |  | dBm/SCS | 1, 2, | -100 |  |  |
|  |  |  | 3 | -97 |  |  |
| Ês/Noc |  | dB | 1, 2, 3 | 12 | -4 | -4 |
| Ês/IotNote3 |  | dB | 1, 2, 3 | 12 | -4 | -4 |
| SS-RSRPNote3 |  | dBm/SCS | 1, 2 | -88 | -104 | -104 |
|  |  |  | 3 | -85 | -101 | -101 |
| IoNote3 |  | dBm/9.36 MHz | 1, 2 | -59.78 | -70.59 | -70.59 |
|  |  | dBm/38.16 MHz | 3 | -53.68 | -64.49 | -64.49 |
| Propagation condition |  |  | 1, 2, 3 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1, 2, 3 | 1x2 Low |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.3.1.6-4: Cell specific test parameters for SA inter-RAT UTRAN FDD handover (Cell 2)

| Parameter | Unit | Cell 2 (UTRA) |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| UTRA RF Channel Number |  | 2 |  |  |
| CPICH_Ec/Ior | dB | -10 |  |  |
| PCCPCH_Ec/Ior | dB | -12 |  |  |
| SCH_Ec/Ior | dB | -12 |  |  |
| PICH_Ec/Ior | dB | -15 |  |  |
| DCH_Ec/Ior | dB | N/A | N/A | Note 1 |
| OCNS_Ec/Ior | dB | -0.941 | 0.941 | Note 2 |
|  | dB | -infinity | -1.8 | -1.8 |
|  | dBm/3,84 MHz | -70 | -70 | -70 |
| CPICH_Ec/Io | dB | -infinity | -14 | -14 |
| Propagation Condition |  | AWGN |  |  |
| Note 1: The DPCH level is controlled by the power control loop Note 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior . |  |  |  |  |

##### A.6.3.1.6.2	Test Requirements

The UE shall start to transmit the UL DPCCH to Cell 2 less than 190 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 50 ms, which is specified in clause 5.3.1.1.1.

Tinterrupt = 140 ms in the test; Tinterrupt is defined in clause 5.3.1.1.2. 	This gives a total of 190 ms.

#### A.6.3.1.7	Intra-frequency synchronous DAPS handover in FR1

##### A.6.3.1.7.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 intra frequency DAPS handover requirements in synchronous scenario specified in clause 6.1.3.2.

##### A.6.3.1.7.2	Test Parameters

Supported test configurations are shown in Table A.6.3.1.7.2-1. Both handover delay and interruption length are tested by using the parameters in Table A.6.3.1.7.2-2, and A.6.3.1.7.2-3.The test consists of five successive time periods, with time durations of T1, T2, T3, T4, and T5 respectively.

Before the start of T1, the UE is connected to the cell1 and not aware of the cell2. The UE shall be configured with periodic CSI reporting for cell1. During T1, the UE does not have any timing information of the cell2.

Starting T2, the cell2 becomes detectable. During T2, the UE performs cell detection and measurements on the cell2 and shall send event report to the network. After receiving the event report A3, the network sends a RRC message implying DAPS handover to the UE.

The start of T3 is the instant when the last TTI containing DAPS handover command is sent to the UE. During T3, UE shall be able to perform random access, DL reception or UL transmission in the cell2 while the DL scheduling and UL feedback in the cell1 shall be avoided. After successful RACH procedure of the cell2, UE is scheduled with PDSCH from cell1 and cell2 in alternative TTIs where both cell1 and cell2 belong to the same TAG. In the end the network sends a RRC message implying cell1 release to the UE. During T3, the handover delay Dhandover1 for target cell addition need to be verified.

The start of T4 is the instant when the last TTI containing cell1 release command is sent to the UE. During T4, the UE shall accomplish the release actions within Dhandover2.

Starting T5, the UE stops sending the periodical CSI report to the cell1.

Table A.6.3.1.7.2-1: Intra-frequency DAPS handover in FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.1.7.2-2: General test parameters synchronous Intra-frequency DAPS handover in FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |
| T4 |  | ms | Dhandover2 | DHandover2 is defined in clause 6.1.3.2.1 |
| T5 |  | ms | 100 |  |

Table A.6.3.1.7.2-3: Cell specific test parameters for NR FR1-FR1 Intra frequency DAPS handover test case

| Parameter |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 - T5 | T1 | T2 - T5 |
| NR RF Channel Number |  |  | 1 |  | 1 |  |
| Duplex mode | Config 1 |  | FDD |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |
| TDD configuration | Config 1 |  | Not Applicable |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |
| BWchannel | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  | Config 2 |  | 10: NRB,c = 52 |  |  |  |
|  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| BWP BW | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  | Config 2 |  | 10: NRB,c = 52 |  |  |  |
|  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| DRX Cycle |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  |  |
| CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |  |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  |  |
| TRS configuration | Config 1 |  | TRS.1.1 FDD |  |  |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |
| CSI-RS configuration for CSI reporting | Config 1 |  | CSI-RS.1.1 FDD |  |  |  |
|  | Config 2 |  | CSI-RS.1.1 TDD |  |  |  |
|  | Config 3 |  | CSI-RS.2.1 TDD |  |  |  |
| reportConfigType |  |  | periodic |  | N/A |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  | N/A |  |
| CSI reporting periodicity | Config 1,2 | slot | 5 |  | N/A |  |
|  | Config 3 |  | 10 |  | N/A |  |
| CSI reporting offset | Config 1,2 | slot | 3 |  | N/A |  |
|  | Config 3 |  | 5 |  | N/A |  |
| SMTC Configuration |  |  | SMTC.1 |  |  |  |
| SSB Configuration | Config 1,2 |  | SSB.1 FR1 |  |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2 | kHz | 15 kHz |  |  |  |
|  | Config 3 |  | 30 kHz |  |  |  |
| PUCCH/PUSCH subcarrier spacing | Config 1,2 | kHz | 15 kHz |  |  |  |
|  | Config 3 |  | 30 kHz |  |  |  |
| PRACH configuration |  |  | FR1 PRACH configuration 1 |  |  |  |
| BWP configuration | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -98 |  |  |  |
| Note2 | Config 1,2 | dBm/SCS | -98 |  |  |  |
|  | Config 3 |  | -95 |  |  |  |
|  |  | dB | 8 | -1.5 | -Infinity | 0.36 |
|  |  | dB | 8 | 8 | -Infinity | 9 |
| SSB_RP | Config 1,2 | dBm/SCS | -90 | -90 | -Infinity | -89 |
|  | Config 3 | dBm/SCS | -87 | -87 | -Infinity | -86 |
| IoNote3 | Config 1,2 | dBm/ 9.36MHz | -61.41 | -58.21 | -61.41 | -58.21 |
|  | Config 3 | dBm/ 38.16MHz | -55.31 | -52.11 | -55.31 | -52.11 |
| Propagation condition |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.3.1.7.3	Test Requirements

The UE shall start to transmit the PRACH to cell 2 less than 72 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The target cell add delay Dhandover1 can be expressed as: TRRC_procedure + Tsearch + TIU + Tprocessing + T∆ + Tmargin, where:

TRRC_procedure  = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tsearch, TIU, Tprocessing, T∆ and Tmargin are defined in clause 6.1.1.2.2.

If the target cell is known, then Tsearch = 0 ms

TIU = 20 ms in the test. TIU is defined in clause 6.1.1.2.2.

T∆ = 20 ms in the test. T∆ is defined in clause 6.1.1.2.2.

Tprocessing = 20 ms in the test. Tprocessing is defined in clause 6.1.1.2.2.

Tmargin = 2 ms in the test. Tmargin is defined in clause 6.1.1.2.2.

This gives a total of 72 ms.

After successful RACH to cell 2 and until the start of time period T4, UE shall be able to receive PDSCH alternatively from cell 1 and cell 2. UE is not expected to transmit UL to both cell 1 and cell 2 in the same TTI.

The UE shall release cell 1 less than Dhandover2 = (TRRC_procedure + Tinterrupt2) from the beginning of time period T4.

NOTE:	Dhandover2 is defined in clause 6.1.3.2.1.

TRRC_procedure  = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt2 is defined in clause 6.1.3.2.2.

UE shall not report CSI to cell 1 during T5.

#### A.6.3.1.8	Intra-frequency asynchronous DAPS handover in FR1

##### A.6.3.1.8.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 intra frequency DAPS handover requirements in asynchronous scenario specified in clause 6.1.3.2.

##### A.6.3.1.8.2	Test Parameters

Supported test configurations are shown in Table A.6.3.1.8.2-1. Both handover delay and interruption length are tested by using the parameters in Table A.6.3.1.8.2-2, and A.6.3.1.8.2-3.

The test consists of five successive time periods, with time durations of T1, T2, T3, T4, and T5 respectively.

Before the start of T1, the UE is connected to the cell1 and not aware of the cell2. The UE shall be configured with periodic CSI reporting for cell1. During T1, the UE does not have any timing information of the cell2.

Starting T2, the cell2 becomes detectable. During T2, the UE performs cell detection and measurements on the cell2 and shall send event report to the network. After receiving the event report A3, the network sends a RRC message implying DAPS handover to the UE.

The start of T3 is the instant when the last TTI containing DAPS handover command is sent to the UE. During T3, UE shall be able to perform random access, DL reception or UL transmission in the cell2 while the DL scheduling and UL feedback in the cell1 shall be avoided. After successful RACH procedure of the cell2, UE is scheduled with PDSCH from cell1 and cell2 in alternative TTIs where both cell1 and cell2 belong to the same TAG. In the end the network sends a RRC message implying cell1 release to the UE. During T3, the handover delay Dhandover1 for target cell addition needs to be verified.

The start of T4 is the instant when the last TTI containing cell1 release command is sent to the UE by cell2. During T4, the UE shall accomplish the release actions within Dhandover2.

Starting T5, the UE stops sending the periodical CSI report to the cell1.

Table A.6.3.1.8.2-1: Intra-frequency DAPS handover in FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.1.8.2-2: General test parameters Intra-frequency asynchronous DAPS handover in FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 7 s | Asynchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |
| T4 |  | ms | Dhandover2 | DHandover2 is defined in clause 6.1.3.2.1 |
| T5 |  | ms | 100 |  |

Table A.6.3.1.8.2-3: Cell specific test parameters for NR FR1-FR1 Intra frequency DAPS handover test case

| Parameter |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 - T5 | T1 | T2 - T5 |
| NR RF Channel Number |  |  | 1 |  | 1 |  |
| Duplex mode | Config 1 |  | FDD |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |
| TDD configuration | Config 1 |  | Not Applicable |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |
| BWchannel | Config 1,2 | MHz | 10: NRB,c = 52 |  |  |  |
|  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| BWP BW | Config 1,2 | MHz | 10: NRB,c = 52 |  |  |  |
|  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| DRX Cycle |  | ms | Not Applicable |  |  |  |
| PDSCH Reference | Config 1 |  | SR.1.1 FDD |  |  |  |
| measurement channel | Config 2 |  | SR.1.1 TDD |  |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  |  |
| CORESET Reference | Config 1 |  | CR.1.1 FDD |  |  |  |
| Channel | Config 2 |  | CR.1.1 TDD |  |  |  |
|  | Config 3 |  | CR.2.1 FDD |  |  |  |
| TRS configuration | Config 1 |  | TRS.1.1 FDD |  |  |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |
| CSI-RS configuration for | Config 1 |  | CSI-RS.1.1 FDD |  |  |  |
| CSI reporting | Config 2 |  | CSI-RS.1.1 TDD |  |  |  |
|  | Config 3 |  | CSI-RS.2.1 TDD |  |  |  |
| reportConfigType |  |  | periodic |  | N/A |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  | N/A |  |
| CSI reporting periodicity | Config 1,2 | slot | 5 |  | N/A |  |
|  | Config 3 |  | 10 |  | N/A |  |
| CSI reporting offset | Config 1,2 | slot | 3 |  | N/A |  |
|  | Config 3 |  | 5 |  | N/A |  |
| SMTC Configuration |  |  | SMTC.1 |  |  |  |
| SSB Configuration | Config 1,2 |  | SSB.1 FR1 |  |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier | Config 1,2 | kHz | 15 |  |  |  |
| spacing | Config 3 |  | 30 |  |  |  |
| PUCCH/PUSCH subcarrier | Config 1,2 | kHz | 15 |  |  |  |
| spacing | Config 3 |  | 30 |  |  |  |
| PRACH configuration |  |  | FR1 PRACH configuration 1 |  |  |  |
| BWP configuration | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -98 |  |  |  |
| Note2 | Config 1,2 | dBm/SCS | -98 |  |  |  |
|  | Config 3 |  | -95 |  |  |  |
|  |  | dB | 8 | -1.5 | -Infinity | 0.36 |
|  |  | dB | 8 | 8 | -Infinity | 9 |
| SSB_RP | Config 1,2 | dBm/SCS | -90 | -90 | -Infinity | -89 |
|  | Config 3 |  | -87 | -87 | -Infinity | -86 |
| IoNote3 | Config 1,2 | dBm/ 9.36MHz | -61.41 | -58.21 | -61.41 | -58.21 |
|  | Config 3 | dBm/ 38.16MHz | -55.31 | -52.11 | -55.31 | -52.11 |
| Propagation condition |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.3.1.8.3	Test Requirements

The UE shall start to transmit the PRACH to cell 2 less than 72 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The target cell add delay Dhandover1 can be expressed as: TRRC_procedure + Tsearch + TIU + Tprocessing + T∆ + Tmargin, where:

TRRC_procedure  = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tsearch, TIU, Tprocessing, T∆ and Tmargin are defined in clause 6.1.1.2.2.

If the target cell is known, then Tsearch = 0 ms

TIU = 20 ms in the test. TIU is defined in clause 6.1.1.2.2.

T∆ = 20 ms in the test. T∆ is defined in clause 6.1.1.2.2.

Tprocessing = 20 ms in the test. Tprocessing is defined in clause 6.1.1.2.2.

Tmargin = 2 ms in the test. Tmargin is defined in clause 6.1.1.2.2.

This gives a total of 72 ms.

After successful RACH to cell 2 and until the start of time period T4, UE shall be able to receive PDSCH alternatively from cell 1 and cell 2. UE is not expected to transmit UL to both cell 1 and cell 2 in the same TTI.

The UE shall release cell 1 less than Dhandover2 = (TRRC_procedure + Tinterrupt2) from the beginning of time period T4.

NOTE:	Dhandover2 is defined in clause 6.1.3.2.1.

TRRC_procedure  = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt2 is defined in clause 6.1.3.2.2.

UE shall not report CSI to cell 1 during T5.

#### A.6.3.1.9	Intra-band inter-frequency synchronous DAPS handover test in SA for FR1

##### A.6.3.1.9.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 intra-band inter-frequency synchronous DAPS handover requirements specified in clause 6.1.3.2.

##### A.6.3.1.9.2	Test Parameters

Supported test configurations are shown in table A.6.3.1.9.2-1. Both handover delay and interruption length are tested by using the parameters in table A.6.3.1.9.2-2, and A.6.3.1.9.2-3.

The test consists of five successive time periods, with time durations of T1, T2, T3, T4 and T5 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2. The UE shall be configured with periodic CSI reporting for cell1. The test scenario comprises of two carriers and one cell on each carrier. Gap pattern ID gp0 as specified in Table 9.1.2-1 is configured before T2 in the test case.

Starting T2, Cell 2 becomes known to the UE. During T2, the UE shall report Event A3. After receiving the Event A3, the test system shall send a RRC message implying DAPS handover to the UE.

T3 is defined as the end of the last TTI containing the RRC message implying DAPS handover. During T3 UE shall be able to perform random access to cell 2. Cell 1 is continuously scheduled in DL during T3. DL schedule and UL feedback to cell 1 shall be avoided when UE is required to perfrom DL reception or UL transmission in PRACH procedure in cell 2, except preamble transmission. At the end of T3 cell 2 shall send an RRC message implying cell 1 release command.

T4 is defined as the end of the last TTI containing the RRC message implying DAPS handover. Cell 2 is continuously scheduled in DL during T4. During T4, the UE shall perform source cell release.

Starting T5, the UE shall stop sending CSI report to the source cell. And the test system shall observe the periodic reporting of CSI for cell 1 during T5.

Table A.6.3.1.9.2-1: Intra-band inter-frequency synchronous DAPS handover in SA for FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.1.9.2-2: General test parameters for intra-band inter-frequency synchronous DAPS  handover test in SA for FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 0 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |
| T4 |  | ms | 10 + Tinterrupt2 | Tinterrupt2 is defined in clause 6.1.3.2.2 Table 6.1.3.2.2-5 |
| T5 |  | ms | 100 |  |

Table A.6.3.1.9.2-3: Cell specific test parameters for intra-band inter-frequency synchronous DAPS handover test in SA for FR1

| Parameter |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 - T5 | T1 | T2 - T5 |
| NR RF Channel Number |  |  | 1 |  | 2 |  |
| Duplex mode | Config 1 |  | FDD |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |
| TDD configuration | Config 1 |  | Not Applicable |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |
| BWchannel | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  | Config 2 |  | 10: NRB,c = 52 |  |  |  |
|  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| BWP BW | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  | Config 2 |  | 10: NRB,c = 52 |  |  |  |
|  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| DRX Cycle |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  |  |
| CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |  |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  |  |
| TRS configuration | Config 1 |  | TRS.1.1 FDD |  |  |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |
| CSI-RS configuration for CSI reporting | Config 1 |  | CSI-RS.1.1 FDD |  |  |  |
|  | Config 2 |  | CSI-RS.1.1 TDD |  |  |  |
|  | Config 3 |  | CSI-RS.2.1 TDD |  |  |  |
| reportConfigType |  |  | periodic |  | N/A |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  | N/A |  |
| CSI reporting periodicity | Config 1,2 | slot | 5 |  | N/A |  |
|  | Config 3 |  | 10 |  | N/A |  |
| CSI reporting offset | Config 1,2 | slot | 3 |  | N/A |  |
|  | Config 3 |  | 5 |  | N/A |  |
| SMTC Configuration |  |  | SMTC.1 |  |  |  |
| SSB Configuration | Config 1,2 |  | SSB.1 FR1 |  |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2 | kHz | 15 kHz |  |  |  |
|  | Config 3 |  | 30 kHz |  |  |  |
| PUCCH/PUSCH subcarrier spacing | Config 1,2 | kHz | 15 kHz |  |  |  |
|  | Config 3 |  | 30 kHz |  |  |  |
| PRACH configuration |  |  | FR1 PRACH configuration 1 |  |  |  |
| BWP configuration | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -98 |  |  |  |
| Note2 | Config 1,2 | dBm/SCS | -98 |  |  |  |
|  | Config 3 |  | -95 |  |  |  |
|  |  | dB | 8 | 8 | -Infinity | 8 |
|  |  | dB | 8 | 8 | -Infinity | 8 |
| SSB_RP | Config 1,2 | dBm/SCS | -90 | -90 | -Infinity | -90 |
|  | Config 3 | dBm/SCS | -87 | -87 | -Infinity | -87 |
| IoNote3 | Config 1,2 | dBm/ 9.36MHz | -61.41 | -61.41 | -70.05 | -61.41 |
|  | Config 3 | dBm/ 38.16MHz | -55.31 | -55.31 | -63.94 | -55.31 |
| Propagation condition |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.3.1.9.3	Test Requirements

The UE shall start to transmit the PRACH to cell 2 less than 72 ms from the beginning of time period T3.

During T3 UE is allowed to cause Tinterrupt1 interruption to cell 1. Tinterrupt1 is defined in clause 6.1.3.2.2 Table 6.1.3.2.2-2. When UE is transmitting PRACH preamble to cell 2, interruption to cell 1 is allowed.

During T4 UE is allowed to cause Tinterrupt2 interruption to cell 1. Tinterrupt2 is defined in clause 6.1.3.2.2 Table 6.1.3.2.2-5.

UE shall finish cell 1 release in T4 and shall not send any CSI reports to cell 1 during T5.

The rate of correct handovers observed during repeated tests shall be at least 90%.

#### A.6.3.1.10	Intra-band inter-frequency asynchronous DAPS handover test in SA for FR1

##### A.6.3.1.10.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 intra-band inter-frequency asynchronous DAPS handover requirements specified in clause 6.1.3.2.

##### A.6.3.1.10.2	Test Parameters

Supported test configurations are shown in table A.6.3.1.10.2-1. Both handover delay and interruption length are tested by using the parameters in table A.6.3.1.10.2-2, and A.6.3.1.10.2-3.

The test consists of five successive time periods, with time durations of T1, T2, T3, T4 and T5 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2. The UE shall be configured with periodic CSI reporting for cell1. The test scenario comprises of two carriers and one cell on each carrier. Gap pattern ID gp0 as specified in Table 9.1.2-1 is configured before T2 in the test case.

Starting T2, Cell 2 becomes known to the UE. During T2, the UE shall report Event A3. After receiving the Event A3, the test system shall send a RRC message implying DAPS handover to the UE.

T3 is defined as the end of the last TTI containing the RRC message implying DAPS handover.During T3 UE shall be able to perform random access to cell 2. Cell 1 is continuously scheduled in DL during T3. DL schedule and UL feedback to cell 1 shall be avoided when UE is required to perfrom DL reception or UL transmission in PRACH procedure in cell 2, except preamble transmission. At the end of T3 cell 2 shall send an RRC message implying cell 1 release command.

T4 is defined as the end of the last TTI containing the RRC message implying DAPS handover. Cell 2 is continuously scheduled in DL during T4. During T4, the UE shall perform source cell release.

Starting T5, the UE shall stop sending CSI report to the source cell. And the test system shall observe the periodic reporting of CSI for cell 1 during T5.

Table A.6.3.1.10.2-1: Intra-band inter-frequency asynchronous DAPS handover in SA for FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.1.10.2-2: General test parameters for intra-band inter-frequency asynchronous DAPS  handover test in SA for FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 10 s | Asynchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |
| T4 |  | ms | 10 + Tinterrupt2 | Tinterrupt2 is defined in clause 6.1.3.2.2 Table 6.1.3.2.2-5 |
| T5 |  | ms | 100 |  |

Table A.6.3.1.10.2-3: Cell specific test parameters for intra-band inter-frequency asynchronous DAPS  handover test in SA for FR1

| Parameter |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 - T5 | T1 | T2 - T5 |
| NR RF Channel Number |  |  | 1 |  | 2 |  |
| Duplex mode | Config 1 |  | FDD |  |  |  |
| TDD configuration | Config 1 |  | Not Applicable |  |  |  |
| BWchannel | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
| BWP BW | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
| DRX Cycle |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |  |  |
| CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |  |  |  |
| TRS configuration | Config 1 |  | TRS.1.1 FDD |  |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |
| CSI-RS configuration for CSI reporting | Config 1 |  | CSI-RS.1.1 FDD |  |  |  |
| reportConfigType |  |  | periodic |  | N/A |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  | N/A |  |
| CSI reporting periodicity | Config 1 | slot | 5 |  | N/A |  |
| CSI reporting offset | Config 1 | slot | 3 |  | N/A |  |
| SMTC Configuration |  |  | SMTC.1 |  |  |  |
| SSB Configuration | Config 1 |  | SSB.1 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing | Config 1 | kHz | 15 kHz |  |  |  |
| PUCCH/PUSCH subcarrier spacing | Config 1 | kHz | 15 kHz |  |  |  |
| PRACH configuration |  |  | FR1 PRACH configuration 1 |  |  |  |
| BWP configuration | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -98 |  |  |  |
| Note2 | Config 1 | dBm/SCS | -98 |  |  |  |
|  |  | dB | 8 | 8 | -Infinity | 8 |
|  |  | dB | 8 | 8 | -Infinity | 8 |
| SSB_RP | Config 1 | dBm/SCS | -90 | -90 | -Infinity | -90 |
| IoNote3 | Config 1 | dBm/ 9.36MHz | -61.41 | -61.41 | -70.05 | -61.41 |
| Propagation condition |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.3.1.10.3	Test Requirements

The UE shall start to transmit the PRACH to cell 2 less than 72 ms from the beginning of time period T3.

During T3 UE is allowed to cause Tinterrupt1 interruption to cell 1. Tinterrupt1 is defined in clause 6.1.3.2.2 Table 6.1.3.2.2-2. When UE is transmitting PRACH preamble to cell 2, interruption to cell 1 is allowed.

During T4 UE is allowed to cause Tinterrupt2 interruption to cell 1. Tinterrupt2 is defined in clause 6.1.3.2.2 Table 6.1.3.2.2-5.

UE shall finish cell 1 release in T4 and shall not send any CSI reports to cell 1 during T5.

The rate of correct handovers observed during repeated tests shall be at least 90%.

#### A.6.3.1.11	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1

##### A.6.3.1.11.1	Test Purpose and Environment

This test is to verify the requirement for the FR1-to-FR1 inter-band inter-frequency synchronous DAPS handover requirements specified in clause 6.1.3.2.

##### A.6.3.1.11.2	Test Parameters

Supported test configurations are shown in table A.6.3.1.11.2-1. Both handover delay and interruption length are tested by using the parameters in table A.6.3.1.11.2-2, A.6.3.1.11.2-3 and A.6.3.1.11.2-4.

The test scenario comprises of two bands each with one cell. The test consists of five successive time periods, with time durations of T1, T2, T3, T4 and T5 respectively.

Before the start of T1, the UE is connected to Cell 1 (source PCell) on radio channel 1 but is not aware of Cell 2 (neighbour cell) on radio channel 2. The UE shall be configured with periodic CSI reporting for cell1. During T1, the UE shall not have any timing information of Cell 2.

Before the start of T2, the UE in the measurement control information that event-triggered reporting with Event A3 is configured for neighbour cell (Cell 2), and the UE is configured with the measurement gaps (gap pattern ID # 0). Starting T2, Cell 2 becomes known to the UE. During T2, the UE shall report Event A3. After receiving the Event A3, the test system shall send a RRC m`essage implying DAPS handover to the UE.

The start of T3 is the instant when the last TTI containing the RRC message implying DAPS handover to Cell 2 (target PCell) is sent to the UE. During T3, the UE shall be able to perform random access to Cell 2. DL schedule and UL feedback to cell 1 shall be avoided when UE is required to perform DL reception or UL transmission in PRACH procedure in cell 2, except preamble transmission. After the RACH procedure is completed, the test system shall send a RRC message to the UE to release Cell 1 (source cell) on radio channel 1.

The start of T4 is the instant when the last TTI containing the RRC message implying source cell release is sent to the UE. During T4, the UE shall perform source cell release.

Starting T5, the UE shall stop sending CSI report to the source cell.

Table A.6.3.1.11.2-1: Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 4 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 5 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 7 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 8 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 9 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.1.11.2-2: General test parameters for inter-band inter-frequency synchronous DAPS handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 | PCell on RF channel number 1 |
|  | Neighbouring cell |  | Cell 2 | Neighbour cell on RF channel number 2 |
| Final condition | Active cell |  | Cell 2 | PCell on RF channel number 2 |
|  | Neighbouring cell |  | Cell 1 | Neighbour cell on RF channel number 1 |
| A3-Offset |  | dB | -6 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  | s | 33 | Synchronous cells |
| DRX |  |  | OFF |  |
| Measurement gap pattern Id |  |  | #0 | Gaps are configured before T2. |
| T1 |  | s | 5 |  |
| T2 |  | s | <5 |  |
| T3 |  | s | <0.5 |  |
| T4 |  | ms | 10+Tinterrupt2 | Tinterrupt2 as defined in Table 6.1.3.2.2-6 for synchronous DAPS HO |
| T5 |  | ms | 100 |  |

Table A.6.3.1.11.2-3: Cell specific test parameters for inter-band inter-frequency synchronous DAPS handover from FR1 to FR1 (Cell 1)

| Parameter |  | Unit | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 – T5 |
| NR RF Channel Number |  |  | 1 |  |
| Duplex mode | Config 1,4,7 |  | FDD |  |
|  | Config 2,3,5,6,8,9 |  | TDD |  |
| TDD configuration | Config 1,4,7 |  | Not Applicable |  |
|  | Config 2,5,8 |  | TDDConf.1.1 |  |
|  | Config 3,6,9 |  | TDDConf.2.1 |  |
| BWchannel | Config 1,4,7 | MHz | 10: NRB,c = 52 |  |
|  | Config 2,5,8 |  | 10: NRB,c = 52 |  |
|  | Config 3,6,9 |  | 40: NRB,c = 106 |  |
| BWP BW | Config 1,4,7 | MHz | 10: NRB,c = 52 |  |
|  | Config 2,5,8 |  | 10: NRB,c = 52 |  |
|  | Config 3,6,9 |  | 40: NRB,c = 106 |  |
| TRS configuration | Config 1,4,7 |  | TRS.1.1 FDD |  |
|  | Config 2,5,8 |  | TRS.1.1 TDD |  |
|  | Config 3,6,9 |  | TRS.1.2 TDD |  |
| DRX Cycle |  | ms | Not Applicable |  |
| PDSCH Reference measurement channel | Config 1,4,7 |  | SR.1.1 FDD |  |
|  | Config 2,5,8 |  | SR.1.1 TDD |  |
|  | Config 3,6,9 |  | SR.2.1 TDD |  |
| CORESET Reference Channel | Config 1,4,7 |  | CR.1.1 FDD |  |
|  | Config 2,5,8 |  | CR.1.1 TDD |  |
|  | Config 3,6,9 |  | CR.2.1 TDD |  |
| OCNG Patterns |  |  | OP.1 |  |
| CSI-RS configuration for CSI reporting | Config 1,4,7 |  | CSI-RS.1.1 FDD |  |
|  | Config 2,5,8 |  | CSI-RS.1.1 TDD |  |
|  | Config 3,6,9 |  | CSI-RS.2.1 TDD |  |
| reportConfigType |  |  | periodic |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| CSI reporting periodicity | Config 1,2,4,5,7,8 | slot | 5 |  |
|  | Config 3,6,9 |  | 10 |  |
| CSI reporting offset | Config 1,2,4,5,7,8 | slot | 3 |  |
|  | Config 3,6,9 |  | 5 |  |
| SMTC Configuration |  |  | SMTC.1 |  |
| SSB Configuration | Config 1,2,4,5,7,8 |  | SSB.1 FR1 |  |
|  | Config 3,6,9 |  | SSB.2 FR1 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,4,5,7,8 | kHz | 15 kHz |  |
|  | Config 3,6,9 |  | 30 kHz |  |
| PUCCH/PUSCH subcarrier spacing | Config 1,2,4,5,7,8 | kHz | 15 kHz |  |
|  | Config 3,6,9 |  | 30 kHz |  |
| PRACH configuration |  |  | FR1 PRACH configuration 2 |  |
| BWP | Initial DL BWP |  | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  | DLBWP.1.3 |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  | ULBWP.1.3 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 |  | dBm/15kHz | -98 | -98 |
| Note2 | Config 1,2,4,5,7,8 | dBm/SCS | -98 | -98 |
|  | Config 3,6,9 |  | -95 | -95 |
|  |  | dB | 4 | 4 |
|  |  | dB | 4 | 4 |
| SSB_RP | Config 1,2,4,5,7,8 | dBm/SCS | -94 | -94 |
|  | Config 3,6,9 | dBm/SCS | -91 | -91 |
| IoNote3 | Config 1,2,4,5,7,8 | dBm/ 9.36MHz | -64.59 | -64.59 |
|  | Config 3,6,9 | dBm/ 38.16MHz | -58.49 | -58.49 |
| Propagation condition |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

Table A.6.3.1.11.2-4: Cell specific test parameters for inter-band inter-frequency synchronous DAPS handover from FR1 to FR1 (Cell 2)

| Parameter |  | Unit | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2-T5 |
| NR RF Channel Number |  |  | 2 |  |
| Duplex mode | Config 1,2,3 |  | FDD |  |
|  | Config 4,5,6,7,8,9 |  | TDD |  |
| TDD configuration | Config 1,2,3 |  | Not Applicable |  |
|  | Config 4,5,6 |  | TDDConf.1.1 |  |
|  | Config 7,8,9 |  | TDDConf.2.1 |  |
| BWchannel | Config 1,2,3 | MHz | 10: NRB,c = 52 |  |
|  | Config 4,5,6 |  | 10: NRB,c = 52 |  |
|  | Config 7,8,9 |  | 40: NRB,c = 106 |  |
| BWP BW | Config 1,2,3 | MHz | 10: NRB,c = 52 |  |
|  | Config 4,5,6 |  | 10: NRB,c = 52 |  |
|  | Config 7,8,9 |  | 40: NRB,c = 106 |  |
| TRS configuration | Config 1,2,3 |  | TRS.1.1 FDD |  |
|  | Config 4,5,6 |  | TRS.1.1 TDD |  |
|  | Config 7,8,9 |  | TRS.1.2 TDD |  |
| DRx Cycle |  | ms | Not Applicable |  |
| PDSCH Reference measurement channel | Config 1,2,3 |  | SR.1.1 FDD |  |
|  | Config 4,5,6 |  | SR.1.1 TDD |  |
|  | Config 7,8,9 |  | SR2.1 TDD |  |
| CORESET Reference Channel | Config 1,2,3 |  | CR.1.1 FDD |  |
|  | Config 4,5,6 |  | CR.1.1 TDD |  |
|  | Config 7,8,9 |  | CR2.1 TDD |  |
| OCNG Patterns |  |  | OCNG pattern 1 |  |
| CSI-RS configuration for CSI reporting | Config 1,2,3 |  | CSI-RS.1.1 FDD |  |
|  | Config 4,5,6 |  | CSI-RS.1.1 TDD |  |
|  | Config 7,8,9 |  | CSI-RS.2.1 TDD |  |
| SMTC Configuration |  |  | SMTC pattern 1 |  |
| SSB Configuration | Config 1,2,3,4,5,6 |  | SSB.1 FR1 |  |
|  | Config 7,8,9 |  | SSB.2 FR1 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,3,4,5,6 | kHz | 15 kHz |  |
|  | Config 7,8,9 |  | 30 kHz |  |
| PUCCH/PUSCH subcarrier spacing | Config 1,2,3,4,5,6 | kHz | 15 kHz |  |
|  | Config 7,8,9 |  | 30 kHz |  |
| PRACH configuration |  |  | FR1 PRACH configuration 2 |  |
| BWP | Initial DL BWP |  | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  | DLBWP.1.3 |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  | ULBWP.1.3 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 |  | dBm/15kHz | -98 | -98 |
| Note2 | Config 1,2,3,4,5,6 | dBm/SCS | -98 | -98 |
|  | Config 7,8,9 |  | -95 | -95 |
|  |  | dB | -Infinity | 4 |
|  |  | dB | -Infinity | 4 |
| SSB_RP | Config 1,2,3,4,5,6 | dBm/SCS | -Infinity | -94 |
|  | Config 7,8,9 | dBm/SCS | -Infinity | -91 |
| IoNote3 | Config 1,2,3,4,5,6 | dBm/ 9.36MHz | -70.05 | -64.59 |
|  | Config 7,8,9 | dBm/ 38.16MHz | -63.94 | -58.49 |
| Propagation condition |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.6.3.1.11.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 72 ms from the beginning of time period T3. During Dhandover1, the interruption on Cell 1 shall not exceed Tinterrupt1 as defined in Table 6.1.3.2.2-3 for synchronous DAPS HO.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay Dhandover1 can be expressed as: TRRC_procedure + TIU + Tprocessing + T∆ + Tmargin, where:

TRRC_procedure = 10 ms and is specified in clause 12 in TS 38.331 [2].

TIU = 20 ms in the test. TIU is defined in clause 6.1.1.2.2.

T∆ = 20 ms in the test. T∆ is defined in clause 6.1.1.2.2.

Tprocessing = 20 ms in the test. Tprocessing is defined in clause 6.1.1.2.2.

Tmargin = 2 ms in the test. Tmargin is defined in clause 6.1.1.2.2.

This gives a total of 72 ms.

The UE shall complete to release Cell 1 less than (10 ms + Tinterrupt2) from the beginning of time period T4. During Dhandover2, the interruption on Cell 2 shall not exceed Tinterrupt2 as defined in Table 6.1.3.2.2-6 for synchronous DAPS HO.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay Dhandover2 can be expressed as: TRRC_procedure + Tinterrupt2, where:

TRRC_procedure = 10 ms and is specified in clause 12 in TS 38.331 [2].

#### A.6.3.1.12	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1

##### A.6.3.1.12.1	Test Purpose and Environment

This test is to verify the requirement for the FR1-to-FR1 inter-band inter-frequency asynchronous DAPS handover requirements specified in clause 6.1.3.2.

##### A.6.3.1.12.2	Test Parameters

Supported test configurations are shown in table A.6.3.1.12.2-1. Both handover delay and interruption length are tested by using the parameters in table A.6.3.1.12.2-2, A.6.3.1.12.2-3 and A.6.3.1.12.2-4.

The test scenario comprises of two bands each with one cell. The test consists of five successive time periods, with time durations of T1, T2, T3, T4 and T5 respectively.

Before the start of T1, the UE is connected to Cell 1 (source PCell) on radio channel 1 but is not aware of Cell 2 (neighbour cell) on radio channel 2. The UE shall be configured with periodic CSI reporting for cell1. During T1, the UE shall not have any timing information of Cell 2.

Before the start of T2, the UE in the measurement control information that event-triggered reporting with Event A3 is configured for neighbour cell (Cell 2), and the UE is configured with the measurement gaps (gap pattern ID # 0). Starting T2, Cell 2 becomes known to the UE. During T2, the UE shall report Event A3. After receiving the Event A3, the test system shall send a RRC message implying DAPS handover to the UE.

The start of T3 is the instant when the last TTI containing the RRC message implying DAPS handover to Cell 2 (target PCell) is sent to the UE. During T3, the UE shall be able to perform random access to Cell 2. DL schedule and UL feedback to cell 1 shall be avoided when UE is required to perform DL reception or UL transmission in PRACH procedure in cell 2, except preamble transmission. After the RACH procedure is completed, the test system shall send a RRC message to the UE to release Cell 1 (source cell) on radio channel 1.

The start of T4 is the instant when the last TTI containing the RRC message implying source cell release is sent to the UE. During T4, the UE shall perform source cell release.

Starting T5, the UE shall stop sending CSI report to the source cell.

Table A.6.3.1.12.2-1: Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 4 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 5 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 7 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 8 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 9 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.1.12.2-2: General test parameters for inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  |  | Cell 1 |  |
|  | Neighbouring cell |  |  | Cell 2 |  |
| Final condition | Active cell |  |  | Cell 2 |  |
| A3-Offset |  |  | dB | -4 |  |
| Hysteresis |  |  | dB | 0 |  |
| Time To Trigger |  |  | s | 0 |  |
| Filter coefficient |  |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  | Config 1,2,4,5 | ms | 0.5 | Asynchronous cells |
|  |  | Config3,6,7,8,9 | ms | 0.25 |  |
| DRX |  |  |  | OFF |  |
| Measurement gap pattern Id |  |  |  | #0 | Gaps are configured before T2. |
| T1 |  |  | s | 5 |  |
| T2 |  |  | s | <5 |  |
| T3 |  |  | s | <0.5 |  |
| T4 |  |  | ms | 10+Tinterrupt2 | Tinterrupt2 as defined in Table 6.1.3.2.2-6 for asynchronous DAPS HO. |
| T5 |  |  | ms | 100 |  |

Table A.6.3.1.12.2-3: Cell specific test parameters for inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1 (Cell 1)

| Parameter |  | Unit | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 – T5 |
| NR RF Channel Number |  |  | 1 |  |
| Duplex mode | Config 1,4,7 |  | FDD |  |
|  | Config 2,3,5,6,8,9 |  | TDD |  |
| TDD configuration | Config 1,4,7 |  | Not Applicable |  |
|  | Config 2,5,8 |  | TDDConf.1.1 |  |
|  | Config 3,6,9 |  | TDDConf.2.1 |  |
| BWchannel | Config 1,4,7 | MHz | 10: NRB,c = 52 |  |
|  | Config 2,5,8 |  | 10: NRB,c = 52 |  |
|  | Config 3,6,9 |  | 40: NRB,c = 106 |  |
| BWP BW | Config 1,4,7 | MHz | 10: NRB,c = 52 |  |
|  | Config 2,5,8 |  | 10: NRB,c = 52 |  |
|  | Config 3,6,9 |  | 40: NRB,c = 106 |  |
| TRS configuration | Config 1,4,7 |  | TRS.1.1 FDD |  |
|  | Config 2,5,8 |  | TRS.1.1 TDD |  |
|  | Config 3,6,9 |  | TRS.1.2 TDD |  |
| DRX Cycle |  | ms | Not Applicable |  |
| PDSCH Reference measurement channel | Config 1,4,7 |  | SR.1.1 FDD |  |
|  | Config 2,5,8 |  | SR.1.1 TDD |  |
|  | Config 3,6,9 |  | SR.2.1 TDD |  |
| CORESET Reference Channel | Config 1,4,7 |  | CR.1.1 FDD |  |
|  | Config 2,5,8 |  | CR.1.1 TDD |  |
|  | Config 3,6,9 |  | CR.2.1 TDD |  |
| OCNG Patterns |  |  | OP.1 |  |
| CSI-RS configuration for CSI reporting | Config 1,4,7 |  | CSI-RS.1.1 FDD |  |
|  | Config 2,5,8 |  | CSI-RS.1.1 TDD |  |
|  | Config 3,6,9 |  | CSI-RS.2.1 TDD |  |
| reportConfigType |  |  | periodic |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| CSI reporting periodicity | Config 1,2,4,5,7,8 | slot | 5 |  |
|  | Config 3,6,9 |  | 10 |  |
| CSI reporting offset | Config 1,2,4,5,7,8 | slot | 3 |  |
|  | Config 3,6,9 |  | 5 |  |
| SMTC Configuration |  |  | SMTC.1 |  |
| SSB Configuration | Config 1,2,4,5,7,8 |  | SSB.1 FR1 |  |
|  | Config 3,6,9 |  | SSB.2 FR1 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,4,5,7,8 | kHz | 15 kHz |  |
|  | Config 3,6,9 |  | 30 kHz |  |
| PUCCH/PUSCH subcarrier spacing | Config 1,2,4,5,7,8 | kHz | 15 kHz |  |
|  | Config 3,6,9 |  | 30 kHz |  |
| PRACH configuration |  |  | FR1 PRACH configuration 2 |  |
| BWP | Initial DL BWP |  | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  | DLBWP.1.3 |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  | ULBWP.1.3 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 |  | dBm/15kHz | -98 | -98 |
| Note2 | Config 1,2,4,5,7,8 | dBm/SCS | -98 | -98 |
|  | Config 3,6,9 |  | -95 | -95 |
|  |  | dB | 4 | 4 |
|  |  | dB | 4 | 4 |
| SSB_RP | Config 1,2,4,5,7,8 | dBm/SCS | -94 | -94 |
|  | Config 3,6,9 | dBm/SCS | -91 | -91 |
| IoNote3 | Config 1,2,4,5,7,8 | dBm/ 9.36MHz | -64.59 | -64.59 |
|  | Config 3,6,9 | dBm/ 38.16MHz | -58.49 | -58.49 |
| Propagation condition |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

Table A.6.3.1.12.2-4: Cell specific test parameters for inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1 (Cell 2)

| Parameter |  | Unit | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2-T5 |
| NR RF Channel Number |  |  | 2 |  |
| Duplex mode | Config 1,2,3 |  | FDD |  |
|  | Config 4,5,6,7,8,9 |  | TDD |  |
| TDD configuration | Config 1,2,3 |  | Not Applicable |  |
|  | Config 4,5,6 |  | TDDConf.1.1 |  |
|  | Config 7,8,9 |  | TDDConf.2.1 |  |
| BWchannel | Config 1,2,3 | MHz | 10: NRB,c = 52 |  |
|  | Config 4,5,6 |  | 10: NRB,c = 52 |  |
|  | Config 7,8,9 |  | 40: NRB,c = 106 |  |
| BWP BW | Config 1,2,3 | MHz | 10: NRB,c = 52 |  |
|  | Config 4,5,6 |  | 10: NRB,c = 52 |  |
|  | Config 7,8,9 |  | 40: NRB,c = 106 |  |
| TRS configuration | Config 1,2,3 |  | TRS.1.1 FDD |  |
|  | Config 4,5,6 |  | TRS.1.1 TDD |  |
|  | Config 7,8,9 |  | TRS.1.2 TDD |  |
| DRx Cycle |  | ms | Not Applicable |  |
| PDSCH Reference measurement channel | Config 1,2,3 |  | SR.1.1 FDD |  |
|  | Config 4,5,6 |  | SR.1.1 TDD |  |
|  | Config 7,8,9 |  | SR2.1 TDD |  |
| CORESET Reference Channel | Config 1,2,3 |  | CR.1.1 FDD |  |
|  | Config 4,5,6 |  | CR.1.1 TDD |  |
|  | Config 7,8,9 |  | CR2.1 TDD |  |
| OCNG Patterns |  |  | OCNG pattern 1 |  |
| CSI-RS configuration for CSI reporting | Config 1,2,3 |  | CSI-RS.1.1 FDD |  |
|  | Config 4,5,6 |  | CSI-RS.1.1 TDD |  |
|  | Config 7,8,9 |  | CSI-RS.2.1 TDD |  |
| SMTC Configuration |  |  | SMTC pattern 1 |  |
| SSB Configuration | Config 1,2,3,4,5,6 |  | SSB.1 FR1 |  |
|  | Config 7,8,9 |  | SSB.2 FR1 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,3,4,5,6 | kHz | 15 kHz |  |
|  | Config 7,8,9 |  | 30 kHz |  |
| PUCCH/PUSCH subcarrier spacing | Config 1,2,3,4,5,6 | kHz | 15 kHz |  |
|  | Config 7,8,9 |  | 30 kHz |  |
| PRACH configuration |  |  | FR1 PRACH configuration 2 |  |
| BWP | Initial DL BWP |  | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  | DLBWP.1.3 |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  | ULBWP.1.3 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 |  | dBm/15kHz | -98 | -98 |
| Note2 | Config 1,2,3,4,5,6 | dBm/SCS | -98 | -98 |
|  | Config 7,8,9 |  | -95 | -95 |
|  |  | dB | -Infinity | 4 |
|  |  | dB | -Infinity | 4 |
| SSB_RP | Config 1,2,3,4,5,6 | dBm/SCS | -Infinity | -94 |
|  | Config 7,8,9 | dBm/SCS | -Infinity | -91 |
| IoNote3 | Config 1,2,3,4,5,6 | dBm/ 9.36MHz | -70.05 | -64.59 |
|  | Config 7,8,9 | dBm/ 38.16MHz | -63.94 | -58.49 |
| Propagation condition |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.6.3.1.12.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 72 ms from the beginning of time period T3. During Dhandover1, the interruption on Cell 1 shall not exceed Tinterrupt1 as defined in Table 6.1.3.2.2-3 for asynchronous DAPS HO.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay Dhandover1 can be expressed as: TRRC_procedure + TIU + Tprocessing + T∆ + Tmargin, where:

TRRC_procedure = 10 ms and is specified in clause 12 in TS 38.331 [2].

TIU = 20 ms in the test. TIU is defined in clause 6.1.1.2.2.

T∆ = 20 ms in the test. T∆ is defined in clause 6.1.1.2.2.

Tprocessing = 20 ms in the test. Tprocessing is defined in clause 6.1.1.2.2.

Tmargin = 2 ms in the test. Tmargin is defined in clause 6.1.1.2.2.

This gives a total of 72 ms.

The UE shall complete to release Cell 1 less than (10 ms + Tinterrupt2) from the beginning of time period T4. During Dhandover2, the interruption on Cell 2 shall not exceed Tinterrupt2 as defined in Table 6.1.3.2.2-6 for asynchronous DAPS HO.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay Dhandover2 can be expressed as: TRRC_procedure + Tinterrupt2, where:

TRRC_procedure = 10 ms and is specified in clause 12 in TS 38.331 [2].

#### A.6.3.1.13	SA NR - E-UTRAN with NR PSCell addition in FR1

##### A.6.3.1.13.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE can make correct inter-RAT E-UTRAN handover with PSCell addition when operating in standalone (SA) operation with PCell in FR1 where target PCell and target PSCell are unknown. This test shall verify the Handover with PSCell from NR SA to EN-DC requirements as specified in clause 6.1.5.1.

The test comprises of two NR carrier and one E-UTRA carrier. There are three cells and one cell on each carrier. Cell 1 is the NR PCell, Cell 2 is an inter-RAT E-UTRAN neighbour cell and Cell 3 is an NR neighbour cell. The test consists of two successive time periods, with time durations of T1 and T2 respectively. At the start of time duration T1, the UE does not have any timing information of Cell 2 and Cell 3. Starting T2, Cell 2 and Cell 3 becomes detectable

A RRC message implying handover with PSCell shall be sent to the UE during period T1. The start of T2 is the instant when the last TTI containing the RRC message implying handover with PSCell is sent to the UE.Before T2, the UE does not have any information of Cell 3. The handover with PSCell message shall contain Cell 2 and Cell 3 as the target cells and the SMTC for Cell 3 is configured in RRCConnectionReconfiguration.

Supported test configurations are shown in table A.6.3.1.13.1-1. General test parameters are provided in Table A.6.3.1.13.1-2. Cell specific test parameters for Cell 1, Cell 2 and Cell 3 are provided in Tables A.6.3.1.13.1-3, A.6.3.1.13.1-4 and A.6.3.1.13.1-5 respectively.

Table A.6.3.1.13.1-1: Supported test configurations for SA inter-RAT E-UTRAN handover tests

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.1.13.1-2: General test parameters for Handover with PSCell from NR SA to EN-DC

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1, 2 | 2 NR carrier frequency is used in the test |
| LTE RF Channel Number |  |  | 3 | 1 E-UTRAN carrier frequency is used in the test |
| Initial conditions | Active cell |  | Cell 1 | NR cell |
|  | Neighbouring cell |  | Cell 2, Cell3 | E-UTRAN cell and NR Cell |
| Final condition | Active cell |  | Cell 2 and Cell 3 |  |
| DRX |  |  | OFF | Non-DRX test |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure |
| Time offset between Cell 1 and Cell 2 |  |  | 3 ms | Asynchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |

Table A.6.3.1.13.1-3: Cell specific test parameters for Handover with PSCell from NR SA to EN-DC (Cell1)

| Parameter |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| RF channel number |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  | 1, 4 | FDD |  |
|  |  |  | 2, 3, 5, 6 | TDD |  |
| TDD Configuration |  |  | 2, 5 | TDDConf.1.1 |  |
|  |  |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |
|  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |
|  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  | 3, 6 | SR.2.1 TDD |  |
| CORSET reference channel |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  | 3, 6 | CR.2.1 TDD |  |
| TRS configuration |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  | 3, 6 | TRS.1.2 TDD |  |
| OCNG patternNote1 |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| BWP | Initial DL BWP |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |
| SMTC configuration |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  | 3, 6 | SSB.2 FR1 |  |
| EPRE ratio of PSS to SSS |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -100 | -100 |
| NocNote2 |  | dBm/SCS | 1, 2, 4, 5 | -100 | -100 |
|  |  |  | 3, 6 | -97 | -97 |
| Ês/Noc |  | dB | 1, 2, 3, 4, 5, 6 | 12 | -4 |
| Ês/IotNote3 |  | dB | 1, 2, 3, 4, 5, 6 | 12 | -4 |
| SS-RSRPNote3 |  | dBm/SCS | 1, 2, 4, 5 | -88 | -104 |
|  |  |  | 3, 6 | -85 | -101 |
| IoNote3 |  | dBm/9.36 MHz | 1, 2, 4, 5 | -59.78 | -70.59 |
|  |  | dBm/38.16 MHz | 3, 6 | -53.68 | -64.49 |
| Propagation condition |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.6.3.1.13.1-4: Cell specific test parameters for Handover with PSCell from NR SA to EN-DC (Cell 2)

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 3 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PRACH ConfigurationNote2 |  | 1, 2, 3 | 4 |  |
|  |  | 4, 5, 6 | 53 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote3 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote3 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote3 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote4 |  |  |  |  |
| OCNG_RBNote4 |  |  |  |  |
| NocNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 78 |
| Ês/IotNote6 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 78 |
| RSRPNote6 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -90 |
| SCH_RPNote6 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -90 |
| IoNote6 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -67.21 +10log(NRB,c/100) | -58.57 +10log(NRB,c/100) |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix Note7 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: PRACH configurations are specified in table 5.7.1-2 and table 5.7.1-3 in TS 36.211 [23]. Note 3: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 4: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 6: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 7: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

Table A.6.3.1.13.1-5: Cell specific test parameters for Handover with PSCell from NR SA to EN-DC (Cell 3)

| Parameter |  | Unit | Configuration | Cell 3 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| RF channel number |  |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  |  | 1, 4 | FDD |  |
|  |  |  | 2, 3, 5, 6 | TDD |  |
| TDD Configuration |  |  | 2, 5 | TDDConf.1.1 |  |
|  |  |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |
|  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |
|  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  | 3, 6 | SR.2.1 TDD |  |
| CORSET reference channel |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  | 3, 6 | CR.2.1 TDD |  |
| TRS configuration |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  | 3, 6 | TRS.1.2 TDD |  |
| PRACH configuration |  |  | 1, 2, 3, 4, 5, 6 | FR1 PRACH configuration 1 |  |
| OCNG patternNote1 |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| BWP | Initial DL BWP |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |
| SMTC configuration |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  | 3, 6 | SSB.2 FR1 |  |
| EPRE ratio of PSS to SSS |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -100 | -85 |
| NocNote2 |  | dBm/SCS | 1, 2, 4, 5 | -100 | -85 |
|  |  |  | 3, 6 | -100 | -82 |
| Ês/Noc |  | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 0 |
| Ês/IotNote3 |  | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 0 |
| SS-RSRPNote3 |  | dBm/SCS | 1, 2, 4, 5 | -Infinity | -85 |
|  |  |  | 3, 6 | -Infinity | -82 |
| IoNote3 |  | dBm/9.36 MHz | 1, 2, 4, 5 | -72.05 | -57 |
|  |  | dBm/38.16 MHz | 3, 6 | -68.96 | -51 |
| Propagation condition |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

##### A.6.3.1.13.2	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 175 ms from the beginning of time period T2.

The UE shall start to transmit the PRACH to Cell 3 less than 270 ms from the beginning of time period T2.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 50 ms and is specified in clause 6.1.5.1.

Tinterrupt = 125 ms in the test; Tinterrupt is defined in clause 6.1.5.1.

The PSCell addition time can be expressed as: TRRC_delay + Tprocessing + Tsearch_HO + Tsearch_PSCell + T∆ + TPSCell_ DU + 2 ms which is defined in clause 6.1.5.1

The rate of correct handovers observed during repeated tests shall be at least 90%.

#### A.6.3.1.14	SA NR - E-UTRAN handover with NR FR1 PScell addition

##### A.6.3.1.14.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE can make correct SA inter-RAT handover from NR to E-UTRAN with FR1 PSCell addition when operating in standalone (SA) operation with PCell in FR1, for the case where the PSCell is known to the UE at the time of addition and SMTC of target known PSCell is not present in RRCConnectionReconfiguration. This test shall verify delay requirements of inter-RAT handover from NR to E-UTRAN and FR1 PSCell addition as specified in clause 6.1.5.

The test comprises of two NR cells and one E-UTRA cell. Cell 1 is the NR PCell, Cell 2 is an inter-RAT E-UTRAN neighbour cell and Cell 3 is the target NR PSCell, on radio channel 1 in FR1, radio channel 2 in E-UTRAN and radio channel 3 in FR1, respectively.

In this test, inter-RAT handover from NR to E-UTRAN and FR1 PSCell addition are performed in parallel processing. The test consists of successive time periods for inter-RAT handover and FR1 PSCell addition with time durations of T1, T2 and T3 respectively.

At the start of time duration T1, the UE does not have any timing information of Cell 2 and Cell 3, and the UE is only monitoring Cell 1. During T1, only Cell1 is known to the UE.

Before the start of T2, the test system shall send measurement control information including measurement gap configuration and event-triggered reporting configuration with event B2 for neighbour Cell 2 and event B1 for Cell 3. Gap pattern configuration with id #0 as specified in Table 9.1.2-1 is configured before T2 begins.

Starting T2, Cell 2 becomes detectable and the UE is expected to detect and send a measurement report, and the Cell 3 (PSCell-to-be) on radio channel 3 becomes known to the UE at the time of addition. The RRC message implying handover with PSCell shall be sent to the UE during period T2 after the UE has reported Event B2 and Event B1. After receiving both Event B2 and Event B1the test system shall send a RRC message to the UE to release the measurement gaps.

The point in time at which the RRC message implying handover with PSCell is received at the UE antenna connector defines the start of period T3 and T3’. The handover with PSCell message shall contain Cell 2 as the target cell and Cell 3 as PSCell-to-be added. The RRC message (to add PSCell) also includes a request for the UE to start periodic CSI reporting for the PSCell after the PSCell has been successfully added.

During T3, the UE shall carry out random access (i.e., transmit the PRACH) towards the Cell 2. Reception by the test system of the PRACH preamble defines the end of T3.

During T3’, the UE shall carry out random access (i.e., transmit the PRACH) towards the Cell 3. Reception by the test system of the PRACH preamble defines the start of period T4’.

During T4’, the UE shall send periodic CSI reports in PSCell and the test system shall observe the periodic reporting of CSI for PSCell.

Supported test configurations are shown in table A.6.3.1.14.1-1. General test parameters are provided in Table A.6.3.1.14.1-2. Cell specific test parameters for NR Cell 1, E-UTRAN PCell Cell 2 and NR PScell Cell 3 are provided in Tables A.6.3.1.14.1-3, A.6.3.1.14.1-4 and A.6.3.1.14.1-5 respectively.

Table A.6.3.1.14.1-1: Supported test configurations for SA inter-RAT E-UTRAN handover with FR1 PSCell addition tests

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD, LTE TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.1.14.1-2: General test parameters for SA inter-RAT E-UTRAN handover with FR1 PSCell addition

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  |  | 1, 3 | 2 NR carrier frequency is used in the test |
| LTE RF Channel Number |  |  |  | 2 | 1 E-UTRAN carrier frequency is used in the test |
| Initial conditions | Active cell |  |  | Cell 1 | NR cell |
|  | Neighbouring cell |  |  | Cell 2, 3 | E-UTRAN cell and NR cell in FR1 |
| Final condition | Active Pcell |  |  | Cell 2 | E-UTRAN cell |
|  | Active PSCell |  |  | Cell 3 | NR cell in FR1 |
| NR measurement quantity |  |  |  | SS-RSRP |  |
| E-UTRAN measurement quantity |  |  |  | RSRP |  |
| Event B1 |  | Hysteresis | dB | 0 | Hysteresis for evaluation of event B1. |
|  |  | Threshold RSRP | dBm | -93 | Actual RSRP threshold for event B1. Needs to take absolute accuracy tolerance in clause 9.1.11.1 into account plus margin. |
|  |  | Time to Trigger | S | 0 |  |
| Event B2 |  | Threshold1 | dBm | As specified in Table A.6.3.1.4-3 | Absolute NR SS-RSRP threshold for event B2 |
|  |  | Threshold2EUTRAN | dBm | -98 | Absolute E-UTRAN RSRP threshold for event B2 |
|  |  | Hysteresis | dB | 0 |  |
|  |  | TimeToTrigger | s | 0 |  |
| Filter coefficient |  |  |  | 0 | L3 filtering is not used |
| DRX |  |  |  | OFF | Non-DRX test |
| Access Barring Information |  |  | - | Not sent | No additional delays in random access procedure |
| Time offset between cell 1 and cell 2 |  |  |  | 3 ms | Asynchronous cells |
| Measurement Gap pattern ID |  |  |  | 0 | As specified in Table 9.1.2-1 |
| T1 |  |  | s | 1 | During this time only Cell 1 is known to UE. |
| T2 |  |  | s | 5 | During this time the UE shall identify Cell 2 and report event B2, and identify Cell 3 and report event B1. |
| T3 |  |  | s | 1 | During this time the UE handovers to Cell 2. |
| T3’ |  |  | s | 0.5 | During this time the UE adds the PSCell (Cell 3). |
| T4’ |  |  | s | 0.5 | During this time the UE sends CSI reports for PSCell (Cell 3). |

Table A.6.3.1.14.1-3: Cell specific test parameters for SA inter-RAT E-UTRA handover with FR1 PSCell addition (NR Cell 1)

| Parameter |  | Unit | Configuration | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 |
| RF channel number |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |  |
| Duplex mode |  |  | 1, 4 | FDD |  |  |
|  |  |  | 2, 3, 5, 6 | TDD |  |  |
| TDD Configuration |  |  | 2, 5 | TDDConf.1.1 |  |  |
|  |  |  | 3, 6 | TDDConf.2.1 |  |  |
| BWchannel |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |  |
|  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |  |
|  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |  |
| PDSCH reference measurement channel |  |  | 1, 4 | SR.1.1 FDD |  |  |
|  |  |  | 2, 5 | SR.1.1 TDD |  |  |
|  |  |  | 3, 6 | SR.2.1 TDD |  |  |
| CORSET reference channel |  |  | 1, 4 | CR.1.1 FDD |  |  |
|  |  |  | 2, 5 | CR.1.1 TDD |  |  |
|  |  |  | 3, 6 | CR.2.1 TDD |  |  |
| TRS configuration |  |  | 1, 4 | TRS.1.1 FDD |  |  |
|  |  |  | 2, 5 | TRS.1.1 TDD |  |  |
|  |  |  | 3, 6 | TRS.1.2 TDD |  |  |
| OCNG patternNote1 |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |  |
| BWP | Initial DL BWP |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |
| SMTC configuration |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |  |
| SSB configuration |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |  |
|  |  |  | 3, 6 | SSB.2 FR1 |  |  |
| b2-Threshold1 |  | dBm | 1, 2, 4, 5 | -96 |  |  |
|  |  |  | 3, 6 | -93 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -100 | -104 | -100 |
| NocNote2 |  | dBm/SCS | 1, 2, 4, 5 | -100 | -104 | -100 |
|  |  |  | 3, 6 | -97 | -101 | -97 |
| Ês/Noc |  | dB | 1, 2, 3, 4, 5, 6 | 12 | 0 | -4 |
| Ês/IotNote3 |  | dB | 1, 2, 3, 4, 5, 6 | 12 | 0 | -4 |
| SS-RSRPNote3 |  | dBm/SCS | 1, 2, 4, 5 | -88 | -104 | -104 |
|  |  |  | 3, 6 | -85 | -101 | -101 |
| IoNote3 |  | dBm/9.36 MHz | 1, 2, 4, 5 | -59.78 | -73.04 | -70.59 |
|  |  | dBm/38.16 MHz | 3, 6 | -53.68 | -66.9448 | -64.49 |
| Propagation condition |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.3.1.14.1-4: Cell specific test parameters for SA inter-RAT E-UTRA handover with FR1 PSCell addition (E-UTRA Cell 2)

| Parameter | Unit | Configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |  |
|  |  | 4, 5, 6 | TDD |  |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |  |
| PRACH ConfigurationNote2 |  | 1, 2, 3 | 4 |  |  |
|  |  | 4, 5, 6 | 53 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote3 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote3 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |  |
| OCNG PatternsNote3 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote4 |  |  |  |  |  |
| OCNG_RBNote4 |  |  |  |  |  |
| NocNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -98 |  |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 8 | 78 |
| Ês/IotNote6 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 78 | 78 |
| RSRPNote6 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -90 | -90 |
| SCH_RPNote6 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -90 | -90 |
| IoNote6 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -67.21 +10log(NRB,c/100) | -58.57 +10log(NRB,c/100) | -58.57 +10log(NRB,c/100) |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix Note7 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: PRACH configurations are specified in table 5.7.1-2 and table 5.7.1-3 in TS 36.211 [23]. Note 3: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 4: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 6: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 7: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |  |

Table A.6.3.1.14.1-5: Cell specific test parameters for SA inter-RAT E-UTRA handover with FR1 PSCell addition (NR Cell 3)

| Parameter | Unit | Config | Test |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3’ | T4’ |  |
| E-UTRA RF Channel Number |  | 1,2,3,4,5,6 | 1 |  |  |  |  |
| NR RF Channel Number |  | 1,2,3,4,5,6 | 2 |  |  |  |  |
| TDD |  | 1,4 | Not Applicable |  |  |  |  |
| configuration |  | 2,5 | TDDConf.1.1 |  |  |  |  |
|  |  | 3,6 | TDDConf.2.1 |  |  |  |  |
| BWchannel | MHz | 1,4 | 10: NRB,c = 52 |  |  |  |  |
|  |  | 2,5 | 10: NRB,c = 52 |  |  |  |  |
|  |  | 3,6 | 40: NRB,c = 106 |  |  |  |  |
| Initial BWP Configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  |  |  |  |
| Dedicated BWP Configuration |  | 1,2,3 | DLBWP.1.1 ULBWP.1.1 |  |  |  |  |
| PDSCH Reference |  | 1,4 | SR.1.1 FDD |  |  |  |  |
| measurement |  | 2,5 | SR.1.1 TDD |  |  |  |  |
| channel |  | 3,6 | SR.2.1 TDD |  |  |  |  |
| RMSI CORESET Reference |  | 1,4 | CR.1.1 FDD |  |  |  |  |
| Channel |  | 2,5 | CR.1.1 TDD |  |  |  |  |
|  |  | 3,6 | CR.2.1 TDD |  |  |  |  |
| Dedicated CORESET Reference |  | 1,4 | CCR.1.1 FDD |  |  |  |  |
| Channel |  | 2,5 | CCR.1.1 TDD |  |  |  |  |
|  |  | 3,6 | CCR.2.1 TDD |  |  |  |  |
| OCNG Patterns |  | 1,2,3,4,5,6 | OP.1 |  |  |  |  |
| SSB configuration |  | 1,2,4,5 | SSB.1 FR1 |  |  |  |  |
|  |  | 3,6 | SSB.2 FR1 |  |  |  |  |
| SMTC configuration |  | 1,2,4,5 | SMTC.1 |  |  |  |  |
|  |  | 3,6 | SMTC.1 |  |  |  |  |
| TRS Configuration |  | 1,4 | TRS.1.1 FDD |  |  |  |  |
|  |  | 2,5 | TRS.1.1 TDD |  |  |  |  |
|  |  | 3,6 | TRS.1.2 TDD |  |  |  |  |
| PRACH configuration on cell2 |  | FR1 PRACH configuration 1 | Captured in A.3.8.2.1 |  |  |  |
| CSI-RS configuration for CSI reporting |  | 1,4 | CSI-RS.1.1 FDD |  |  |  |
|  |  | 2,5 | CSI-RS.1.1 TDD |  |  |  |
|  |  | 3,6 | CSI-RS.2.1 TDD |  |  |  |
| reportConfigType |  | 1,2,3,4,5,6 | periodic |  |  |  |
| reportQuantity |  | 1,2,3,4,5,6 | cri-RI-PMI-CQI |  |  |  |
| CSI reporting periodicity | slot | 1,2,4,5 | 5 |  |  |  |
|  |  | 3,6 | 10 |  |  |  |
| CSI reporting offset | slot | 1,2,4,5 | 2 |  |  |  |
|  |  | 3,6 | 4 |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS | dB | 1,2,3,4,5,6 | 0 |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1,2,3,4,5,6 | N/A | -85 |  |  |
| Note2 | dBm/SCS | 1,2,4,5 | N/A | -85 |  |  |
|  |  | 3,6 | N/A | -82 |  |  |
|  |  | 1,2,3,4,5,6 | -infinity | 0 |  |  |
|  |  | 1,2,3,4,5,6 | -infinity | 0 |  |  |
| SS-RSRPNote3 | dBm/SCS | 1,2,4,5 | -infinity | -85 |  |  |
|  |  | 3,6 | -infinity | -82 |  |  |
| IoNote3 | dBm/9.36MHz | 1,2,4,5 | N/A | -57 |  |  |
|  | dBm/38.1MHz | 3,6 | N/A | -51 |  |  |
| Propagation condition |  | 1,2,3,4,5,6 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |

##### A.6.3.1.14.2	Test Requirements

In this test, the UE shall start to transmit the PRACH to E-UTRA Cell 2 less than 85 ms Note1 from the beginning of time period T3.

The UE shall transmit the PRACH to PSCell no later than 117 ms Note2 from the start of T3’. The UE shall send at least one CSI report for PSCell with non-zero CQI index during T4’. The UE shall periodically send CSI reports for PSCell after the UE has sent first CQI report with non-zero CQI index during T4’.

The above test requirements shall be fulfilled in order of T1, T2, T3 for the observed inter-RAT handover delay from NR to E-UTRAN to be counted as correct, and in order of T1, T2, T3‘, T4‘ for the observed PSCell addition delay to be counted as correct.

The rate of correct handovers and correct PSCell addition delay during repeated tests shall be at least 90%.

NOTE1:	The handover delay can be expressed as specified in clause 6.1.5.2:

DHOwithPSCell_PCell = RRC procedure delay + Tinterrupt,

Where RRC procedure delay = 50 ms, Tinterrupt = Tsearch_HO + TIU + Tprocessing is defined in clause 6.1.5.2.1, where

Tsearch = 0 ms

TIU = 10 ms,

Tprocessing = 25ms

Note2: The PSCell addition delay can be expressed as follows as specified in clause 6.1.5.2.2:

DHOwithPSCell_PSCell = TRRC_delay + Tprocessing + Tsearch_HO + Tsearch_PSCell + T∆ + TPSCell_ DU + 2 ms

Where:

TRRC_delay = 50 ms

Tprocessing = 25ms

Tsearch_HO  = 0

Tsearch_PSCell = 0

T∆ = 20ms

TPSCell_ DU = 1*10+10 = 20ms

### A.6.3.2	RRC Connection Mobility Control

#### A.6.3.2.1	SA: RRC Re-establishment

##### A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1

A.6.3.2.1.1.1	Test Purpose and Environment

The purpose is to verify that the NR intra-frequency RRC re-establishment delay in FR1 with known target cell is within the specified limits. These tests will verify the requirements in clause 6.2.1.

The test parameters are given in table A.6.3.2.1.1.1-1, table A.6.3.2.1.1.1-2 and table A.6.3.2.1.1.1-3 below. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, is deactivated. The time period T3 starts after the occurrence of the radio link failure.

Table A.6.3.2.1.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.3.2.1.1.1-2: General test parameters for NR intra-frequency RRC Re-establishment test case in FR1

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell1 |  |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| Final condition | Active cell |  | 1, 2, 3 | Cell2 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| N310 |  | - | 1, 2, 3 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1, 2, 3 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1, 2, 3 | 0 | Radio link failure timer; |
| T311 |  | ms | 1, 2, 3 | 3000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC.2 |  |
|  |  |  | 2 | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2, 3 | OFF |  |
| PRACH configuration |  |  | 1, 2, 3 | FR1 PRACH configuration 1 | Table A.3.8.2.1-1 |
| T1 |  | s | 1, 2, 3 | 5 |  |
| T2 |  | ms | 1, 2, 3 | 240 | Time for the UE to detect RLF (Summation of TEvaluate_out_SSB defined in clause 8.1 in TS 38.133, T310 and the period for UE turns off transmitter defined in clause 8.1.5 in TS 38.133 ) |
| T3 |  | s | 1, 2, 3 | 2 |  |

Table A.6.3.2.1.1.1-3: Cell specific test parameters for NR intra-frequency RRC Re-establishment test case in FR1

| Parameter | Unit | Test configuration | Cell 1 |  |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 |  | T2 |  | T3 | T1 | T2 |  |  | T3 |
| TDD configuration |  | 1 | N/A |  |  |  |  | N/A |  |  |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  |  | TDDConf.1.1 |  |  |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  |  | TDDConf.2.1 |  |  |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |  |  |  | SR.1.1 FDD |  |  |  |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  | SR.1.1 TDD |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  | SR.2.1 TDD |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |  |  |  | CR.1.1 FDD |  |  |  |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |  | CR.1.1 TDD |  |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |  | CR.2.1 TDD |  |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  |  |  |  | CCR.1.1 FDD |  |  |  |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |  | CCR.1.1 TDD |  |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |  | CCR.2.1 TDD |  |  |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  |  |  |  | OP.1 defined in A.3.2.1 |  |  |  |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  |  |  |  | TRS.1.1 FDD |  |  |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |  | TRS.1.1 TDD |  |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |  | TRS.1.2 TDD |  |  |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  |  |  |  | DLBWP.0.1 |  |  |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  |  |  |  | ULBWP.0.1 |  |  |  |  |
| Active DL BWP confgiuration |  | 1, 2, 3 | DLBWP.1.1 | N/A |  | N/A |  | N/A |  | N/A | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 | N/A |  | N/A |  | N/A |  | N/A | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  |  |  | SSB |  |  |  |  |
|  | dB | 1 | 1.54 |  | -infinity |  | -infinity | -3.79 | 4 |  |  | 4 |
|  |  | 2 |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |
|  | dB | 1 | 7 |  | -infinity |  | -infinity | 4 | 4 |  |  | 4 |
|  |  | 2 |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -91 |  | -infinity |  | -infinity | -94 | -94 |  |  | -94 |
|  |  | 2 | -91 |  | -infinity |  | -infinity | -94 | -94 |  |  | -94 |
|  |  | 3 | -88 |  | -infinity |  | -infinity | -91 | -91 |  |  | -91 |
| Io | dBm/9.36 MHz | 1 | -60.74 |  | -64.59 |  | -64.59 | -60.74 | -64.59 |  |  | -64.59 |
|  | dBm/9.36 MHz | 2 | -60.74 |  | -64.59 |  | -64.59 | -60.74 | -64.59 |  |  | -64.59 |
|  | dBm/38.16 MHz | 3 | -54.65 |  | -58.50 |  | -58.50 | -54.65 | -58.50 |  |  | -58.50 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |  |

A.6.3.2.1.1.2	Test Requirements

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to a known NR intra frequency cell shall be less than 1.6 s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE_re-establish_delay.

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

TUE_re-establish_delay=50 ms+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

Nfreq = 1

Tidentify_intra_NR = 200 ms

TSI = 1280 ms; it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target intra-frequency NR cell.

TPRACH = 15 ms; it is the additional delay caused by the random access procedure.

This gives a total of 1545 ms, allow 1.6 s in the test case.

##### A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1

A.6.3.2.1.2.1	Test Purpose and Environment

The purpose is to verify that the NR inter-frequency RRC re-establishment delay in FR1 without known target cell is within the specified limits. These tests will verify the requirements in clause 6.2.1.

The test parameters are given in table A.6.3.2.1.2.1-1, table A.6.3.2.1.2.1-2 and table A.6.3.2.1.2.1-3 below. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, becomes inactive. The time period T3 starts after the occurrence of the radio link failure. During T1, the UE shall be configured with the carrier frequency of cell 2 (with RF Channel Number #2) to ensure that the UE has the context of the carrier frequency of cell 2 by the end of T1.

Table A.6.3.2.1.2.1-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.6.3.2.1.2.1-2: General test parameters for NR inter-frequency RRC Re-establishment test case in FR1

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell1 |  |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| Final condition | Active cell |  | 1, 2, 3 | Cell2 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1, 2 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| N310 |  | - | 1, 2, 3 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1, 2, 3 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1, 2, 3 | 0 | Radio link failure timer; |
| T311 |  | ms | 1, 2, 3 | 5000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC.2 |  |
|  |  |  | 2 | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2, 3 | OFF |  |
| PRACH configuration |  |  | 1, 2, 3 | FR1 PRACH configuration 1 | Table A.3.8.2.1-1 |
| T1 |  | s | 1, 2, 3 | 5 |  |
| T2 |  | ms | 1, 2, 3 | 240 | Time for the UE to detect RLF (Summation of TEvaluate_out_SSB defined in clause 8.1 in TS 38.133, T310 and the period for UE turns off transmitter defined in clause 8.1.5 in TS 38.133 ) |
| T3 |  | s | 1, 2, 3 | 5 |  |

Table A.6.3.2.1.2.1-3: Cell specific test parameters for NR inter-frequency RRC Re-establishment test case in FR1

| Parameter | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 |  | T2 | T3 | T1 | T2 |  |  | T3 |
| RF Channel Number |  | 1, 2, 3 | 1 |  |  |  | 2 |  |  |  |  |
| TDD configuration |  | 1 | N/A |  |  |  | N/A |  |  |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  | TDDConf.1.1 |  |  |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  | TDDConf.2.1 |  |  |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |  |  | SR.1.1 FDD |  |  |  |  |
|  |  | 2 | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  | SR.2.1 TDD |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |  |  | CR.1.1 FDD |  |  |  |  |
|  |  | 2 | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  | CR.2.1 TDD |  |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  |  |  | CCR.1.1 FDD |  |  |  |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |  |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  | CCR.2.1 TDD |  |  |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  |  |  | OP.1 defined in A.3.2.1 |  |  |  |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  |  |  | TRS.1.1 FDD |  |  |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  |  |  | DLBWP.0.1 |  |  |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  |  |  | ULBWP.0.1 |  |  |  |  |
| Active DL BWP confgiuration |  | 1, 2, 3 | DLBWP.1.1 | N/A |  | N/A | N/A |  | N/A | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 | N/A |  | N/A | N/A |  | N/A | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  |  | SSB |  |  |  |  |
|  | dB | 1 | 4 |  | -infinity | -infinity | -infinity | -infinity |  |  | 7 |
|  |  | 2 |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |
|  | dB | 1 | 4 |  | -infinity | -infinity | -infinity | -infinity |  |  | 7 |
|  |  | 2 |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -94 |  | -infinity | -infinity | -infinity | -infinity |  |  | -91 |
|  |  | 2 | -94 |  | -infinity | -infinity | -infinity | -infinity |  |  | -91 |
|  |  | 3 | -91 |  | -infinity | -infinity | -infinity | -infinity |  |  | -88 |
| Io | dBm/9.36 MHz | 1 | -64.59 |  | -70. 05 | -70. 05 | -70. 05 | -70. 05 |  |  | -62.26 |
|  | dBm/9.36 MHz | 2 | -64.59 |  | -70. 05 | -70. 05 | -70. 05 | -70.05 |  |  | -62.26 |
|  | dBm/38.16 MHz | 3 | -58.50 |  | -63.94 | -63.94 | -63.94 | -63.94 |  |  | -56.15 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |

A.6.3.2.1.2.2	Test Requirements

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NR inter frequency cell shall be less than 3 s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE_re-establish_delay.

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

TUE_re-establish_delay=50 ms+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

Nfreq = 2

Tidentify_intra_NR = 800 ms

Tidentify_inter_NR = 800 ms

TSI = 1280 ms; it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target inter-frequency NR cell.

TPRACH = 15 ms; it is the additional delay caused by the random access procedure.

This gives a total of 2945 ms, allow 3 s in the test case.

##### A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing

A.6.3.2.1.3.1	Test Purpose and Environment

The purpose is to verify that the NR intra-frequency RRC re-establishment delay in FR1 without serving cell timing is within the specified limits. These tests will verify the requirements in clause 6.2.1.

The test parameters are given in table A.6.3.2.1.3.1-1, table A.6.3.2.1.3.1-2 and table A.6.3.2.1.3.1-3 below. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, is deactivated. The time period T3 starts after the occurrence of the radio link failure.

Table A.6.3.2.1.3.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.3.2.1.3.1-2: General test parameters for NR intra-frequency RRC Re-establishment test case in FR1

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell1 |  |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| Final condition | Active cell |  | 1, 2, 3 | Cell2 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| N310 |  | - | 1, 2, 3 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1, 2, 3 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1, 2, 3 | 6000 | Radio link failure timer configured by RLF-TimersAndConstants |
| T311 |  | ms | 1, 2, 3 | 3000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC.2 |  |
|  |  |  | 2 | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2, 3 | OFF |  |
| PRACH configuration |  |  | 1, 2, 3 | FR1 PRACH configuration 1 | Table A.3.8.2.1-1 |
| T1 |  | s | 1, 2, 3 | 5 |  |
| T2 |  | s | 1, 2, 3 | 6.24 | Time for the UE to detect RLF (Summation of TEvaluate_out_SSB defined in clause 8.1 in TS 38.133, T310 and the period for UE turns off transmitter defined in clause 8.1.5 in TS 38.133 ) |
| T3 |  | s | 1, 2, 3 | 3 |  |

Table A.6.3.2.1.3.1-3: Cell specific test parameters for NR intra-frequency RRC Re-establishment test case in FR1

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1 | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
|  |  | 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
|  |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
|  |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| TRS Configuration |  | 1 | TRS.1.1.FDD |  |  | TRS.1.1.FDD |  |  |
|  |  | 2 | TRS.1.1.TDD |  |  | TRS.1.1.TDD |  |  |
|  |  | 3 | TRS.1.2.TDD |  |  | TRS.1.2.TDD |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  | SSB |  |  |
|  | dB | 1 | 4 | -infinity | -infinity | -infinity | -infinity | 4 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
|  | dB | 1 | 4 | -infinity | -infinity | -infinity | -infinity | 4 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -94 | -infinity | -infinity | -infinity | -infinity | -94 |
|  |  | 2 | -94 | -infinity | -infinity | -infinity | -infinity | -94 |
|  |  | 3 | -91 | -infinity | -infinity | -infinity | -infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.59 | -infinity | -infinity | -infinity | -infinity | -64.59 |
|  | dBm/9.36 MHz | 2 | -64.59 | -infinity | -infinity | -infinity | -infinity | -64.59 |
|  | dBm/38.16 MHz | 3 | -58.50 | -infinity | -infinity | -infinity | -infinity | -58.50 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

A.6.3.2.1.3.2	Test Requirements

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NR intra frequency cell without serving cell timing shall be less than 2.2 s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE_re-establish_delay.

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

TUE_re-establish_delay=50 ms+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

Nfreq = 1

Tidentify_intra_NR = 800 ms

TSI = 1280 ms; it is the time required for receiving all the relevant system information as defined in TS 38.331 [2] for the target intra-frequency NR cell.

TPRACH = 15 ms; it is the additional delay caused by the random access procedure.

This gives a total of 2145 ms, allow 2.2 s in the test case.

#### A.6.3.2.2	Random Access

##### A.6.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone

A.6.3.2.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the behavior of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits. This test will verify the requirements in Clause 6.2.2.2 and Clause 7.1.2 in an AWGN model.

For this test one cell is used and configured as PCell in FR1. Supported test parameters are shown in Table A.6.3.2.2.1.1-1. UE capable of SA with PCell in FR1 needs to be tested by using the parameters in Table A.6.3.2.2.1.1-2.

Table A.6.3.2.2.1.1-1: Supported test configurations for contention based random access test in FR1 for NR standalone

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |

Table A.6.3.2.2.1.1-2: General test parameters for contention based random access test in FR1 for NR Standalone

| Parameter |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- |
| SSB Configuration |  | Config 1 |  | SSB pattern 1 in FR1 | As defined in A.3.10, except for number of SSBs per SS-burst and SS/PBCH block index as below |
|  |  | Config 2 |  | SSB pattern 2 in FR1 |  |
| Number of SSBs per SS-burst |  |  |  | 2 | Different from the definition in A.3.10 |
| SS/PBCH block index |  |  |  | 0,1 | Different from the definition in A.3.10 |
| Duplex Mode for Cell 1 |  | Config 1 |  | FDD |  |
|  |  | Config 2 |  | TDD |  |
| TDD Configuration |  | Config 2 |  | TDDConf.2.1 |  |
| CSI-RS for tracking |  | Config 1 |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.2 TDD |  |
| OCNG Pattern Note 1 |  |  |  | OP.1 | As defined in A.3.2.1. |
| PDSCH parameters Note 4 |  | Config 1 |  | SR.1.1 FDD | As defined in A.3.1.1. |
|  |  | Config 2 |  | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.1.1 FDD |  |
|  |  | Config 2 |  | CCR.2.1 TDD |  |
| NR RF Channel Number |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  | dB |  |  |
| SSB with index 0 |  |  | dB | 3 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  |  | Config 1 | dBm/15kHz | -98 |  |
|  |  | Config 2 |  | -101 |  |
|  |  |  | dB | 3 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -95 |  |
| SSB with index 1 |  |  | dB | -17 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  |  | Config 1 | dBm/15kHz | -98 |  |
|  |  | Config 2 |  | -101 |  |
|  |  |  | dB | -17 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -115 |  |
| Io Note 2 |  | Config 1 | dBm | -65.3/9.36MHz | For symbols without SSB index 1 |
|  |  | Config 2 |  | -62.2/38.16MHz |  |
| ss-PBCH-BlockPower |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [2]. |
| Configured UE transmitted power () |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1. |
| PRACH Configuration |  |  |  | FR1 PRACH configuration 1 | As defined in A.3.8. |
| Propagation Condition |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: Void Note 4: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |  |  |

A.6.3.2.2.1.2	Test Requirements

Contention based random access is triggered by not explicitly assigning a random access preamble via dedicated signalling in the downlink.

A.6.3.2.2.1.2.1	Random Access Preamble Transmission

To test the UE behavior specified in Clause 6.2.2.2.1.1 the System Simulator shall receive the Random Access Preamble which belongs to one of the Random Access Preambles associated with the SSB with index 0, which has SS-RSRP above the configured rsrp-ThresholdSSB.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2.2. The power of the first preamble shall be 22  dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

A.6.3.2.2.1.2.2	Random Access Response Reception

To test the UE behavior specified in Clause 6.2.2.2.1.2 the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. In response to the first 4 preambles, the System Simulator shall transmit a Random Access Response not corresponding to the transmitted Random Access Preamble.

The UE may stop monitoring for Random Access Response(s) and shall transmit the msg3 if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2.2. The power of the first preamble shall be 22  dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

A.6.3.2.2.1.2.3	No Random Access Response Reception

To test the UE behavior specified in clause 6.2.2.2.1.3 the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. The System Simulator shall not respond to the first 4 preambles.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2.2. The power of the first preamble shall be 22  dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

A.6.3.2.2.1.2.4	Receiving an UL grant for msg3 retransmission

To test the UE behavior specified in clause 6.2.2.2.1.4 the System Simulator shall provide an UL grant for msg3 retransmission following a successful Random Access Response.

The UE shall re-transmit the msg3 upon the reception of an UL grant for msg3 retransmission.

A.6.3.2.2.1.2.5	Reception of an Incorrect Message over Temporary C-RNTI

To test the UE behavior specified in Clause 6.2.2.2.1.5 the System Simulator shall send a message addressed to the temporary C-RNTI with a UE Contention Resolution Identity included in the MAC control element not matching the CCCH SDU transmitted in msg3 uplink message.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires unless the received message includes a UE Contention Resolution Identity MAC control element and the UE Contention Resolution Identity included in the MAC control element matches the CCCH SDU transmitted in the uplink message.

A. 6.3.2.2.1.2.6	Reception of a Correct Message over Temporary C-RNTI

To test the UE behavior specified in Clause 6.2.2.2.1.5 the System Simulator shall send a message addressed to the temporary C-RNTI with a UE Contention Resolution Identity included in the MAC control element matching the CCCH SDU transmitted in the msg3 uplink message.

The UE shall send ACK if the Contention Resolution is successful.

A.6.3.2.2.1.2.7	Contention Resolution Timer expiry

To test the UE behavior specified in Clause 6.2.2.2.1.6 the System Simulator shall not send a response to a msg3.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if the Contention Resolution Timer expires.

##### A.6.3.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone

A.6.3.2.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the behavior of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits. This test will verify the requirements in Clause 6.2.2.2 and Clause 7.1.2 in an AWGN model.

For this test one cell is used and configured as PCell in FR1. Supported test parameters are shown in Table A.6.3.2.2.2.1-1. UE capable of SA with PCell in FR1 needs to be tested by using the parameters in Table A.6.3.2.2.2.1-2 for SSB-based non-contention based random access test (Test 1) and CSI-RS-based non-contention based random access test (Test 2). Test 2 is only applicable to UE which supports csi-RSRP-AndRSRQ-MeasWithSSB or csi-RSRP-AndRSRQ-MeasWithoutSSB.

Table A.6.3.2.2.2.1-1: Supported test configurations for non-contention based random access test in FR1 for NR standalone

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |

Table A.6.3.2.2.2.1-2: General test parameters for non-contention based random access test in FR1 for NR Standalone

| Parameter |  |  | Unit | Test-1 | Test-2 | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| SSB Configuration |  | Config 1 |  | SSB pattern 1 in FR1 | SSB pattern 1 in FR1 | As defined in A.3.10, except for number of SSBs per SS-burst and SS/PBCH block index as below |
|  |  | Config 2 |  | SSB pattern 2 in FR1 | SSB pattern 2 in FR1 |  |
| Number of SSBs per SS-burst |  |  |  | 2 | 2 | Different from the definition in A.3.10 |
| SS/PBCH block index |  |  |  | 0,1 | 0,1 | Different from the definition in A.3.10 |
| CSI-RS Configuration |  | Config 1 |  | N/A | CSI-RS.1.1 FDD | As defined in A.3.1.4 |
|  |  | Config 2 |  |  | CSI-RS.2.1 TDD |  |
| Duplex Mode for Cell 1 |  | Config 1 |  | FDD | FDD |  |
|  |  | Config 2 |  | TDD | TDD |  |
| TDD Configuration |  | Config 2 |  | TDDConf.2.1 | TDDConf.2.1 |  |
| CSI-RS for tracking |  | Config 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.2 TDD | TRS.1.2 TDD |  |
| OCNG Pattern Note 1 |  |  |  | OP.1 | OP.1 | As defined in A.3.2.1. |
| PDSCH parameters Note 4 |  | Config 1 |  | SR.1.1 FDD | SR.1.1 FDD | As defined in A.3.1.1. |
|  |  | Config 2 |  | SR.2.1 TDD | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 TDD | CR.1.1 TDD |  |
|  |  | Config 2 |  | CR.2.1 TDD | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.1.1 TDD | CCR.1.1 TDD |  |
|  |  | Config 2 |  | CCR.2.1 TDD | CCR.2.1 TDD |  |
| NR RF Channel Number |  |  |  | 1 | 1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  | dB |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  | dB |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  | dB |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  | dB |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  | dB |  |  |  |
| SSB with index 0 |  |  | dB | 3 | 3 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  |  | Config 1 | dBm/15kHz | -98 | -98 |  |
|  |  | Config 2 |  | -101 | -101 |  |
|  |  |  | dB | 3 | 3 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -95 | -95 |  |
| SSB with index 1 |  |  | dB | -17 | -17 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  |  | Config 1 | dBm/15kHz | -98 | -98 |  |
|  |  | Config 2 |  | -101 | -101 |  |
|  |  |  | dB | -17 | -17 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -115 | -115 |  |
| Io Note 2 |  | Config 1 | dBm | -65.3/9.36MHz | -65.3/9.36MHz | For symbols without SSB index 1 |
|  |  | Config 2 |  | -62.2/38.16MHz | -62.2/38.16MHz |  |
| ss-PBCH-BlockPower |  |  | dBm/ SCS | -5 | -5 | As defined in clause 6.3.2 in TS 38.331 [2]. |
| Configured UE transmitted power () |  |  | dBm | 23 | 23 | As defined in clause 6.2.4 in TS 38.101-1. |
| PRACH Configuration |  |  |  | FR1 PRACH configuration 2 | FR1 PRACH configuration 3 | As defined in A.3.8.2. |
| Propagation Condition |  |  | - | AWGN | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: Void Note 4: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |  |  |  |

A.6.3.2.2.2.2	Test Requirements

Non-Contention based random access is triggered by explicitly assigning a random access preamble via dedicated signalling in the downlink. In the test, the non-contention based random access procedure is not initialized for Other SI requested from UE or beam failure recovery.

A.6.3.2.2.2.2.1	SSB-based Random Access Preamble Transmission

In Test-1, to test the UE behavior specified in Clause 6.2.2.2.2.1 for SSB-based Random Access Preamble tranmsision, with the contention-free Random Access Resources and the contention-free PRACH occasions associated with SSBs configured, the System Simulator shall receive the Random Access Preamble which has the Preamble Index associated with the SSB with index 0.

In addition, the System Simulator shall receive the Random Access Preamble on the PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with index 0, and the selected PRACH occasion shall belongs to the PRACH occassions permitted by the restrictions given by the ra-ssb-OccasionMaskIndex.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2.2. The power of the first preamble shall be 22  dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

A.6.3.2.2.2.2.2	CSI-RS-based Random Access Preamble Transmission

In Test-2, to test the UE behavior specified in Clause 6.2.2.2.2.1 for CSI-RS-based Random Access Preamble tranmsision, with the contention-free Random Access Resources and the contention-free PRACH occasions associated with CSI-RSs configured, the System Simulator shall receive the Random Access Preamble which has the Preamble Index associated with the CSI-RS configured.

In addition, the System Simulator shall receive the Random Access Preamble on the PRACH occasion which belongs to the PRACH occasions corresponding to the CSI-RS configured, and the selected PRACH occasion shall belongs to the PRACH occassions permitted by the restrictions given by the ra-OccasionList.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2.2. The power of the first preamble shall be 22  dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

A.6.3.2.2.2.2.3	Random Access Response Reception

To test the UE behavior specified in Clause 6.2.2.2.2.2 the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. In response to the first 4 preambles, the System Simulator shall transmit a Random Access Response not corresponding to the transmitted Random Access Preamble.

The UE may stop monitoring for Random Access Response(s) if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2.2. The power of the first preamble shall be 22  dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

A.6.3.2.2.2.2.4	No Random Access Response Reception

To test the UE behavior specified in clause 6.2.2.2.2.3 the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. The System Simulator shall not respond to the first 4 preambles.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window configured in RACH-ConfigCommon.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2.2. The power of the first preamble shall be 22  dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

##### A.6.3.2.2.3	2-step RA type contention based random access test in FR1 for NR standalone

A.6.3.2.2.3.1	Test Purpose and Environment

The purpose of this test is to verify that the behavior of the 2-step RA type random access procedure is according to the requirements and that the MsgA PRACH, MsgA PUSCH power settings and timing are within specified limits. This test will verify the requirements in Clause 6.2.2.3 and Clause 7.1.2 in an AWGN model.

For this test one cell is used and configured as PCell in FR1. Supported test parameters are shown in Table A.6.3.2.2.3.1-1. UE capable of SA with PCell in FR1 needs to be tested by using the parameters in Table A.6.3.2.2.3.1-2.

Table A.6.3.2.2.3.1-1: Supported test configurations for 2-step RA type contention based random access with successRAR test in FR1 for NR standalone

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |

Table A.6.3.2.2.3.1-2: General test parameters for 2-step RA type contention based random access with successRAR test in FR1 for NR standalone

| Parameter |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- |
| SSB Configuration |  | Config 1 |  | SSB pattern 1 in FR1 | As defined in A.3.10, except for number of SSBs per SS-burst and SS/PBCH block index as below |
|  |  | Config 2 |  | SSB pattern 2 in FR1 |  |
| Number of SSBs per SS-burst |  |  |  | 2 | Different from the definition in A.3.10 |
| SS/PBCH block index |  |  |  | 0,1 | Different from the definition in A.3.10 |
| Duplex Mode for Cell 2 |  | Config 1 |  | FDD |  |
|  |  | Config 2 |  | TDD |  |
| TDD Configuration |  | Config 2 |  | TDDConf.2.1 |  |
| OCNG Pattern Note 1 |  |  |  | OP.1 | As defined in A.3.2.1. |
| PDSCH parameters Note 3 |  | Config 1 |  | SR.1.1 FDD | As defined in A.3.1.1. |
|  |  | Config 2 |  | SR.2.1 TDD |  |
| NR RF Channel Number |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  | dB |  |  |
| SSB with index 0 |  |  | dB | 3 | Power of SSB with index 0 is set to be above configured msgA-RSRP-ThresholdSSB |
|  |  | Config 1 | dBm/15kHz | -98 |  |
|  |  | Config 2 |  | -101 |  |
|  |  |  | dB | 3 |  |
|  | SS-RSRP |  | dBm/ SCS | -95 |  |
| SSB with index 1 |  |  | dB | -17 | Power of SSB with index 1 is set to be below configured msgA-RSRP-ThresholdSSB |
|  |  | Config 1 | dBm/15kHz | -98 |  |
|  |  | Config 2 |  | -101 |  |
|  |  |  | dB | -17 |  |
|  | SS-RSRP |  | dBm/ SCS | -115 |  |
| Io Note 2 |  | Config 1 | dBm | -65.3/9.36MHz | For symbols without SSB index 1 |
|  |  | Config 2 |  | -62.2/38.16MHz |  |
| ss-PBCH-BlockPower |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [2]. |
| Configured UE transmitted power () |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1. |
| MsgA Configuration |  |  |  | FR1 MsgA configuration 1 | As defined in A.3.20.2.1. |
| msgA-RSRP-ThresholdSSB |  |  | dBm | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [2]. |
| Propagation Condition |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |  |  |

A.6.3.2.2.3.2	Test Requirements

Contention based random access is triggered by not explicitly assigning a random access preamble via dedicated signalling in the downlink.

A.6.3.2.2.3.2.1	MsgA Transmission

To test the UE behavior specified in Clause 6.2.2.3.1.1 the System Simulator shall receive the MsgA with a preamble which belongs to one of the Random Access Preambles associated with the SSB with index 0, which has SS-RSRP above the configured msgA-RSRP-ThresholdSSB.

In addition, the power applied to all MsgA transmissions shall be in accordance with what is specified in Clause 6.2.2.3. The power of the first MsgA preamble transmission shall be -22 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The power of the first MsgA PUSCH transmission shall be  0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional MsgA transmissions shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all MsgA transmissions shall be within the accuracy specified in Clause 7.1.2.

A.6.3.2.2.3.2.2	MsgB Reception

To test the UE behavior specified in Clause 6.2.2.3.1.2 the System Simulator shall transmit a MsgB containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. In response to the first 4 preambles, the System Simulator shall transmit a MsgB not corresponding to the transmitted Random Access Preamble.

The UE may stop monitoring for MsgB(s) and shall transmit an ACK if the MsgB with a successRAR contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble and if the Contention Resolution is successful.

The UE may stop monitoring for MsgB(s) and shall transmit the msg3 if the MsgB with a fallbackRAR contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS 38.321 [7], and transmit with the calculated MsgA PRACH and MsgA PUSCH transmission power when the backoff time expires if all received MsgB(s) contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

In addition, the power applied to all MsgA transmissions shall be in accordance with what is specified in Clause 6.2.2.3. The power of the first preamble shall be -22 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The power of the first MsgA PUSCH transmission shall be  0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional MsgA transmissions shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all MsgA transmissions shall be within the accuracy specified in Clause 7.1.2.

A.6.3.2.2.3.2.3	No MsgB Reception

To test the UE behavior specified in clause 6.2.2.3.1.3 the System Simulator shall transmit a MsgB containing a successRAR message and a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. The System Simulator shall not respond to the first 4 preambles.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS 38.321 [7], and transmit with the calculated MsgA PRACH and MsgA PUSCH transmission power when the backoff time expires if no MsgB  is received within the MsgB Response window.

In addition, the power applied to all MsgA transmissions shall be in accordance with what is specified in Clause 6.2.2.3. The power of the first preamble shall be -22 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The power of the first MsgA PUSCH transmission shall be  0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional MsgA transmissions shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all MsgA transmissions shall be within the accuracy specified in Clause 7.1.2.

##### A.6.3.2.2.4	2-step RA type non-contention based test in FR1 for NR standalone

A.6.3.2.2.4.1	Test Purpose and Environment

The purpose of this test is to verify that the behavior of the random access procedure is according to the requirements and that the MsgA PRACH, MsgA PUSCH power settings and timing are within specified limits. This test will verify the requirements in Clause 6.2.2.3 and Clause 7.1.2 in an AWGN model.

For this test one cell is used and configured as PCell in FR1. Supported test parameters are shown in Table A.6.3.2.2.4.1-1. UE capable of SA with PCell in FR1 needs to be tested by using the parameters in Table A.6.3.2.2.4.1-2.

Table A.6.3.2.2.4.1-1: Supported test configurations for non-contention based random access test in FR1 for NR standalone

| Config | Description |
| --- | --- |
| 1 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |

Table A.6.3.2.2.4.1-2: General test parameters for non-contention based random access test in FR1 for NR Standalone

| Parameter |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- |
| SSB Configuration |  | Config 1 |  | SSB pattern 2 in FR1 | As defined in A.3.10, except for number of SSBs per SS-burst and SS/PBCH block index as below |
| Number of SSBs per SS-burst |  |  |  | 2 | Different from the definition in A.3.10 |
| SS/PBCH block index |  |  |  | 0,1 | Different from the definition in A.3.10 |
| Duplex Mode for Cell 1 |  | Config 1 |  | TDD |  |
| TDD Configuration |  | Config 1 |  | TDDConf.2.1 |  |
| OCNG Pattern Note 1 |  |  |  | OP.1 | As defined in A.3.2.1. |
| PDSCH parameters Note 4 |  | Config 1 |  | SR.2.1 TDD | As defined in A.3.1.1. |
| NR RF Channel Number |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  | dB |  |  |
| msgA-RSRP-ThresholdSSB |  |  | dBm | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [2]. |
| SSB with index 0 |  |  | dB | 3 | Power of SSB with index 0 is set to be above configured msgA-RSRP-ThresholdSSB |
|  |  | Config 1 | dBm/15kHz | -101 |  |
|  |  |  | dB | 3 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -95 |  |
| SSB with index 1 |  |  | dB | -17 | Power of SSB with index 1 is set to be below configured msgA-RSRP-ThresholdSSB |
|  |  | Config 1 | dBm/15kHz | -101 |  |
|  |  |  | dB | -17 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -115 |  |
| Io Note 2 |  | Config 1 | dBm | -62.2/38.16MHz | For symbols without SSB index 1 |
| ss-PBCH-BlockPower |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [2]. |
| Configured UE transmitted power () |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1. |
| MsgA Configuration |  |  |  | FR1 MsgA configuration 2 | As defined in A.3.20.2.2. |
| Propagation Condition |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |  |  |

A.6.3.2.2.4.2	Test Requirements

Non-Contention based random access is triggered by explicitly assigning a random access preamble via dedicated signalling in the downlink. In the test, the non-contention based random access procedure is not initialized for Other SI requested from UE or beam failure recovery.

A.6.3.2.2.4.2.1	MsgA Transmission

To test the UE behavior specified in Clause 6.2.2.3.2.1, with the contention-free Random Access Resources and the contention-free PRACH occasions associated with SSBs configured, the System Simulator shall receive the MsgA with a preamble which belongs to one of the Random Access Preambles associated with the SSB with index 0.

In addition, the System Simulator shall receive the MsgA PRACH on the PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with index 0, and the selected PRACH occasion shall belongs to the PRACH occasions permitted by the restrictions given first by the msgA-SSB-SharedRO-MaskIndex if configured, or next by the ra-ssb-OccasionMaskIndex if configured.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2.3. The power of the first preamble shall be -22 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The power of the first MsgA PUSCH transmission shall be 0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional MsgA transmissions shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all MsgA transmissions shall be within the accuracy specified in Clause 7.1.2.

A.6.3.2.2.4.2.2	MsgB Reception

To test the UE behavior specified in Clause 6.2.2.3.2.2 the System Simulator shall transmit a MsgB containing a fallbackRAR containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. In response to the first 4 preambles, the System Simulator shall transmit a MsgB not corresponding to the transmitted Random Access Preamble.

The UE may stop monitoring for MsgB(s) and shall transmit the msg3 containing the payload of MsgA PUSCH if the MsgB with a fallbackRAR contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble. The UE shall monitor contention resolution as described in clause 8.2A in TS 38.213 [3].

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS 38.321 [7], and transmit with the calculated MsgA PRACH and MsgA PUSCH transmission power when the backoff time expires if all received MsgB’s contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2.3. The power of the first preamble shall be -22 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The power of the first MsgA PUSCH transmission shall be 0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all MsgA and msg3 transmissions shall be within the accuracy specified in Clause 7.1.2.

A.6.3.2.2.4.2.3	No MsgB Reception

To test the UE behavior specified in clause 6.2.2.3.2.3 the System Simulator shall transmit a MsgB containing a successRAR message and a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. The System Simulator shall not respond to the first 4 preambles.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS 38.321 [7], and transmit with the calculated MsgA transmission power when the backoff time expires if no MsgB  is received within the MsgB Response window.

In addition, the power applied to all MsgA transmissions shall be in accordance with what is specified in Clause 6.2.2.3. The power of the first preamble shall be -22 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The power of the first MsgA PUSCH transmission shall be 0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional MsgA transmissions shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all MsgA transmissions shall be within the accuracy specified in Clause 7.1.2.

#### A.6.3.2.3	SA: RRC Connection Release with Redirection

##### A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1

A.6.3.2.3.1.1	Test Purpose and Environment

This test is to verify RRC connection release with redirection from NR to NR requirements specified in clause 6.2.3.2.1.

A.6.3.2.3.1.2	Test Parameters

Supported test configurations are shown in table A.6.3.2.3.1.2-1. The time delay is tested by using the parameters in table A.6.3.2.3.1.2-2, and A.6.3.2.3.1.2-3.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. The RRCRelease message shall be sent to the UE during period T1 and the start of T2 is the instant when the last TTI containing the RRC message is sent to the UE. Prior to time duration T2, the UE shall not have any timing information of Cell 2. Cell 2 is powered up at the beginning of the T2. Cell 1 and Cell 2 belong to different tracking areas.

Table A.6.3.2.3.1.2-1: Redirection from NR to NR test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.2.3.1.2-2: General test parameters for Redirection from NR to NR test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 2.3 |  |

Table A.6.3.2.3.1.2-3: Cell specific test parameters for Redirection from NR to NR test case

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 2 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |
| SSB Configuration |  | Config 1 |  | SSB.1 FR1 |  |  |  |  |  |  |
|  |  | Config 2 |  | SSB.1 FR1 |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |
| CSI-RS for tracking |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |
| SMTC configuration |  | Config 1,2 |  | SMTC.1 FR1 |  |  |  |  |  |  |
|  |  | Config 3 |  | SMTC.2 FR1 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |  |  |  |
|  |  |  | dB | 4 | 4 |  | -infinity |  | 4 |  |
|  |  |  | dB | 4 | 4 |  | -infinity |  | 4 |  |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -64.59 | -64.59 |  | -70.05 |  | -64.59 |  |
|  | Config 3 |  | dBm/ 38.16MHz | -58.49 | -58.49 |  | -63.94 |  | -58.49 |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

A.6.3.2.3.1.3	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 2240 ms from the beginning of time period T2.

The rate of correct RRC connection release redirection to NR observed during repeated tests shall be at least 90%.

NOTE:	The redirection delay can be expressed as:

Tconnection_release_redirect_NR = TRRC_procedure_delay + Tidentify-NR + TSI-NR + TRACH,

where:

TRRC_procedure_delay = 110 msin the test.

Tidentify-NR = 680 ms in the test.

TSI-NR = 1280 ms, it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target NR cell.

TRACH = 170 ms in the test.

This gives a total of 2240 ms.

##### A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN

A.6.3.2.3.2.1	Test Purpose and Environment

This test is to verify RRC connection release with redirection from NR to E-UTRAN requirements specified in clause 6.2.3.2.2.

A.6.3.2.3.2.2	Test Parameters

Supported test configurations are shown in table A.6.3.2.3.2.2-1. The time delay is tested by using the parameters in table A.6.3.2.3.2.2-2, A.6.3.2.3.2.2-3 and A.6.3.2.3.2.2-4.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. The RRCRelease message shall be sent to the UE during period T1 and the start of T2 is the instant when the last TTI containing the RRC message is sent to the UE. Prior to time duration T2, the UE shall not have any timing information of Cell 2. Cell 2 is powered up at the beginning of the T2.

Table A.6.3.2.3.2.2-1: Redirection from NR to E-UTRAN test configurations

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.2.3.2.2-2: General test parameters for Redirection from NR to E-UTRAN test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 2.3 |  |

Table A.6.3.2.3.2.2-3: Cell specific test parameters for Redirection from NR to E-UTRAN (cell 1)

| Parameter |  |  | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |
| RF Channel Number |  |  |  | 1 |  |  |
| Duplex mode |  | Config 1,4 |  | FDD |  |  |
|  |  | Config 2,3,5,6 |  | TDD |  |  |
| SSB Configuration |  | Config 1 |  | SSB.1 FR1 |  |  |
|  |  | Config 2 |  | SSB.1 FR1 |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |
| CSI-RS for tracking |  | Config 1 |  | TRS.1.1 FDD |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |
| TDD configuration |  | Config 1,4 |  | Not Applicable |  |  |
|  |  | Config 2,5 |  | TDDConf.1.1 |  |  |
|  |  | Config 3,6 |  | TDDConf.2.1 |  |  |
| BWchannel |  | Config 1,4 | MHz | 10: NRB,c = 52 |  |  |
|  |  | Config 2,5 |  | 10: NRB,c = 52 |  |  |
|  |  | Config 3,6 |  | 40: NRB,c = 106 |  |  |
| BWP BW |  | Config 1,4 | MHz | 10: NRB,c = 52 |  |  |
|  |  | Config 2,5 |  | 10: NRB,c = 52 |  |  |
|  |  | Config 3,6 |  | 40: NRB,c = 106 |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  | SR.1.1 FDD |  |  |
|  |  | Config 2,5 |  | SR.1.1 TDD |  |  |
|  |  | Config 3,6 |  | SR2.1 TDD |  |  |
| CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD |  |  |
|  |  | Config 2,5 |  | CR.1.1 TDD |  |  |
|  |  | Config 3,6 |  | CR2.1 TDD |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |
| SMTC configuration |  | Config 1,2,4,5 |  | SMTC.1 FR1 |  |  |
|  |  | Config 3,6 |  | SMTC.2 FR1 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4,5 | kHz | 15 kHz |  |  |
|  |  | Config 3,6 |  | 30 kHz |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2,4,5 | kHz | 15 kHz |  |  |
|  |  | Config 3,6 |  | 30 kHz |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |
| Note2 | Config 1,2,4,5 |  | dBm/SCS | -98 |  |  |
|  | Config 3,6 |  |  | -95 |  |  |
|  |  |  | dB | 4 | 4 |  |
|  |  |  | dB | 4 | 4 |  |
| IoNote3 | Config 1,2,4,5 |  | dBm/ 9.36MHz | -64.59 | -64.59 |  |
|  | Config 3,6 |  | dBm/ 38.16MHz | -58.49 | -58.49 |  |
| Propagation condition |  |  | - | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.3.2.3.2.2-4: Cell specific test parameters for Redirection from NR to E-UTRAN (cell 2)

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PRACH ConfigurationNote2 |  | 1, 2, 3 | 4 |  |
|  |  | 4, 5, 6 | 53 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote3 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote3 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote3 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote4 |  |  |  |  |
| OCNG_RBNote4 |  |  |  |  |
| NocNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 4 |
| Ês/IotNote6 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 4 |
| RSRPNote6 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -94 |
| SCH_RPNote6 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -94 |
| IoNote6 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -70.22 | -64.76 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: PRACH configurations are specified in table 5.7.1-2 and table 5.7.1-3 in TS 36.211 [23]. Note 3: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 4: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 6: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 7: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

A.6.3.2.3.2.3	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 2205 ms from the beginning of time period T2.

The rate of correct RRC connection release redirection to E-UTRAN observed during repeated tests shall be at least 90%.

NOTE:	The redirection delay can be expressed as:

Tconnection_release_redirect_E-UTRA = TRRC_procedure_delay + Tidentify-E-UTRA + TSI-E-UTRA + TRACH,

where:

TRRC_procedure_delay = 110 ms  in the test.

Tidentify-E-UTRA = 800 ms in the test.

TSI-E-UTRA = 1280 ms, it is the time required for receiving all the relevant system information as defined in TS 36.331 for the target E-UTRA cell.

TRACH = 15 ms in the test.

This gives a total of 2205 ms.

#### A.6.3.3 Conditional handover

#### A.6.3.3.1	Intra-frequency conditional handover from FR1 to FR1

##### A.6.3.3.1.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 intra frequency conditional handover requirements specified in clause 6.1.4.2.

##### A.6.3.3.1.2	Test Parameters

Supported test configurations are shown in table A.6.3.3.1.2-1. Both conditional handover delay and interruption length are tested by using the parameters in table A.6.3.3.1.2-2, and A.6.3.3.1.2-3.

The test consists of two successive time periods, with time durations of T1 and  T2 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2.

NR shall configure a condition implying handover to cell 2 during T1, at a time earlier than TRRC before the beginning of T2.

Table A.6.3.3.1.2-1: Intra-frequency conditional handover from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.3.1.2-2: General test parameters Intra-frequency conditional handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset in condition |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH configuration index |  |  | FR1 PRACH configuration 1 | As specified in table Table 6.3.3.2-3 in TS 38.211 [6] |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 2 |  |

Table A.6.3.3.1.2-3: Cell specific test parameters for NR FR1-FR1 Intra frequency conditional handover test case

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 1 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC pattern 1 |  |  |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |  |  |  |
|  |  |  | dB | 8 | -3.3 |  | -Infinity |  | 2.36 |  |
|  |  |  | dB | 8 | 8 |  | -Infinity |  | 11 |  |
| SSB_RP | Config 1,2 |  | dBm/SCS | -90 | -90 |  | -Infinity |  | -87 |  |
|  | Config 3 |  | dBm/SCS | -87 | -87 |  | -Infinity |  | -84 |  |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -61.41 | -57.06 |  | -61.41 |  | -57.06 |  |
|  | Config 3 |  | dBm/ 38.16MHz | -55.31 | -50.96 |  | -55.31 |  | -50.96 |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

##### A.6.3.3.1.3 Test Requirements

TRRC + TEvent_DU occurs during T1 as the handover condition becomes satisfied at the start of T2. The test shall verify that there are no interruptions during T1.

The UE shall start to transmit the PRACH to Cell 2 less than Tmeasure + Tinterrupt + TCHO_execution = 800 +62 +10=872ms from the start of T2 and the interruption during T2 shall not exceeed Tinterrupt=Tprocessing + TIU + T∆ + Tmargin =40+20+2 = 62ms

#### A.6.3.3.2	Inter-frequency conditional handover from FR1 to FR1

##### A.6.3.3.2.1	Test Purpose and Environment

This test is to verify the requirement for the NR conditional FR1-NR FR1 inter frequency conditional handover requirements specified in clause 6.1.4.2.

##### A.6.3.3.2.2	Test Parameters

Supported test configurations are shown in table A.6.3.3.2.2-1. Both conditional handover delay and interruption length are tested by using the parameters in table A.6.3.3.2.2-2, and A.6.3.3.2.2-3.

The test scenario comprises of two carriers and one cell on each carrier Gap pattern ID gp0 is configured in the test case. The test consists of two successive time periods, with time durations of T1, T2 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. NR shall configure a condition implying handover to cell 2 during T1, at a time earlier than TRRC before the beginning of T2.  At the start of T2, cell 2 becomes detectable and meets the handover condition.

Table A.6.3.3.2.2-1: Inter-frequency handover from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.3.2.2-2: General test parameters Inter-frequency handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset in handover condition |  | dB | -4 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| T1 |  | s | 5 |  |
| T2 |  | s | 2 |  |

Table A.6.3.3.2.2-3: Cell specific test parameters for NR FR1-FR1 Inter frequency handover test case

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 2 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| Gap pattern ID |  |  |  | gp0 |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC pattern 1 |  |  |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |
| BWP |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  | -98 |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  | -98 |  |  |  |
|  | Config 3 |  |  | -95 |  |  | -95 |  |  |  |
|  |  |  | dB | 4 | 4 |  | -Infinity |  | 5 |  |
|  |  |  | dB | 4 | 4 |  | -Infinity |  | 5 |  |
| SSB_RP | Config 1,2 |  | dBm/SCS | -94 | -94 |  | -Infinity |  | -93 |  |
|  | Config 3 |  | dBm/SCS | -91 | -91 |  | -Infinity |  | -90 |  |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -64.59 | -64.59 |  | -70.05 |  | -63.85 |  |
|  | Config 3 |  | dBm/ 38.16MHz | -58.49 | -58.49 |  | -63.94 |  | -57.75 |  |
| Propagation condition |  |  | - | AWGN |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

##### A.6.3.3.2.3 Test Requirements

TRRC + TEvent_DU occurs during T1 as the handover condition becomes satisfied at the start of T2. The test shall verify that there are no interruptions during T1.

The UE shall start to transmit the PRACH to Cell 2 less than Tmeasure + Tinterrupt + TCHO_execution =920 +62 +10=992 ms from the start of T2 and the interruption during T2 shall not exceeed Tinterrupt=Tprocessing + TIU + T∆ + Tmargin =40+20+2 = 62ms excluding any transmissions which do not occur due to measurement gaps.

Inter-frequency CHO FR1-FR1 920 (Tmeasure)+62 (Tinterrupt)+10 (TCHO_execution) = 992 62 m

## A.6.4	Timing

### A.6.4.1	UE transmit timing

#### A.6.4.1.1	NR UE Transmit Timing Test for FR1

##### A.6.4.1.1.1	Test Purpose and environment

The purpose of this test is to verify that the UE can follow frame timing change of the connected gNodeb and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.1.2.

Supported test configurations are shown in Table A.6.4.1.1.1-1.

Table A.6.4.1.1.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NR TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

For this test a single NR cell is used. Table A.6.4.1.1.1-2 defines the parameters to be configured and strength of the transmitted signals. The transmit timing is verified by the UE transmitting SRS using the configuration defined in Table A.6.4.1.1.1-3.

Table A.6.4.1.1.1-2: Cell Specific Test Parameters for UL Transmit Timing test

| Parameter | Unit | Config | Test1 |  | Test2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| SSB ARFCN |  | 1,2,3 | 1 |  | 1 |  |
| TDD configuration |  | 1 | Not Applicable |  |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |  |  |
|  |  | 2 | 10: NRB,c = 52 |  |  |  |
|  |  | 3 | 40: NRB,c = 106 |  |  |  |
| Initial BWP Configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  |  |  |
| Dedicated BWP Configuration |  | 1,2,3 | DLBWP.1.1 ULBWP.1.1 |  |  |  |
| DRx Cycle | ms | 1,2,3 | N/A |  |  | DRX.8Note5 |
| PDSCH Reference measurement channel |  | 1 | SR.1.1 FDD |  |  |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  | 1 | CR.1.1 FDD |  |  |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  | 1 | CCR.1.1 FDD |  |  |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1,2,3 | OP.1 |  |  |  |
| SSB configuration |  | 1,2 | SSB.1 FR1 |  |  |  |
|  |  | 3 | SSB.2 FR1 |  |  |  |
| SMTC Configuration |  | 1,2 | SMTC.1 |  |  |  |
|  |  | 3 | SMTC.2 |  |  |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| EPRE ratio of PSS to SSS | dB | 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1,2,3 | -98 |  | -98 |  |
| Note2 | dBm/SCS | 1,2 | -98 |  | -98 |  |
|  |  | 3 | -95 |  | -95 |  |
|  |  | 1,2,3 | 3 |  | 3 |  |
|  |  | 1,2,3 | 3 |  | 3 |  |
| SS-RSRPNote3 | dBm/SCS | 1,2 | -95 |  | -95 |  |
|  |  | 3 | -92 |  | -92 |  |
| IoNote3 | dBm/9.36MHz | 1,2 | -65.2 |  | -65.2 |  |
|  | dBm/38.1MHz | 3 | -59.2 |  | -59.2 |  |
| Propagation condition |  | 1,2,3 | AWGN |  |  |  |
| SRS Config |  | 1,2 | SRSConf.1Note6 | SRSConf.3Note6 |  |  |
|  |  | 3 | SRSConf.1Note6 | SRSConf.2Note6 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: DRx related parameters are given in Table A.3.3.8-1 Note 6: SRS configs are given in Table A.6.4.1.1.1-3 |  |  |  |  |  |  |

Table A.6.4.1.1.1-3: SRS Configuration for Timing Accuracy Test

|  | Field | SRSConf.1 | SRSConf.2 | SRSConf.3 | Comments |
| --- | --- | --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 0 | 0 |  |
|  | srs-ResourceIdList | 0 | 0 | 0 |  |
|  | resourceType | Periodic | Periodic | Periodic |  |
|  | Usage | Codebook | Codebook | Codebook |  |
| SRS-Resource | SRS-ResourceId | 0 | 0 | 0 |  |
|  | nrofSRS-Ports | Port1 | Port1 | Port1 |  |
|  | transmissionComb | n2 | n2 | n2 |  |
|  | combOffset-n2 | 0 | 0 | 0 |  |
|  | cyclicShift-n2 | 0 | 0 | 0 |  |
|  | resourceMapping startPosition | 0 | 0 | 0 |  |
|  | resourceMapping nrofSymbols | n1 | n1 | n1 |  |
|  | resourceMapping repetitionFactor | n1 | n1 | n1 |  |
|  | freqDomainPosition | 0 | 0 | 0 |  |
|  | freqDomainShift | 0 | 0 | 0 |  |
|  | freqHopping c-SRS | 14 for test configuration 1,2 25 for test configuration 3 | 25 | 14 | Matches NRB,c |
|  | freqHopping b-SRS | 0 | 0 | 0 |  |
|  | freqHopping b-hop | 0 | 0 | 0 |  |
|  | groupOrSequenceHopping | Neither | Neither | Neither |  |
|  | resourceType | Periodic | Periodic | Periodic |  |
|  | periodicityAndOffset-p | sl1, 0 | sl640, 0 | sl320, 0 | Offset to align with DRx periodicity |
|  | sequenceId | 0 | 0 | 0 | Any 10 bit number |

Table A.6.4.1.1.1-4: Void

##### A.6.4.1.1.2	Test requirements

The test sequence shall be carried out in RRC_CONNECTED for every test case.

Following will be the test sequence for this test

1) Setup NR PCell according to parameters given in Table A.6.4.1.1.1-1.

2)	After connection set up with the cell, the test equipment will verify that the timing of the NR cell is within (NTA + NTA_offset) ×Tc ± Te of the first detected path of DL SSB.

a.	The NTA offset value (in Tc units) is 25600

b.	The Te values depend on the DL and UL SCS for which the test is being run and are given in Table 7.1.2-1

3)	The test system shall adjust the timing of the DL path by values given in Table A.6.4.1.1.2-1

Table A.6.4.1.1.2-1: Adjustment Value for DL Timing

| SCS of SSB signals (KHz) | Adjustment Value |  |
| --- | --- | --- |
|  | Test1 | Test2 |
| 15 | +64*64Tc | +32*64Tc |
| 30 | +32*64Tc | +16*64Tc |

4)	The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in clause 7.1.2 Table 7.1.2.1-1 until the UE transmit timing offset is within (NTA + NTA_offset) ×Tc ± Te respective to the first path (in time) of DL SSB used by the UE to determine downlink timing is received from the reference cell at the UE antenna.  Skip this step for test 2 with DRX configured.

5)	The test system shall verify that the UE transmit timing offset stays within (NTA + NTA_offset) ×Tc ± Te of the first path (in time) of DL SSB used by the UE to determine downlink timing is received from the reference cell at the UE antenna. For Test 2 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment

### A.6.4.2	UE timer accuracy

### A.6.4.3	Timing advance

#### A.6.4.3.1	SA FR1 timing advance adjustment accuracy

##### A.6.4.3.1.1	Test Purpose and Environment

The purpose of the test is to verify UE Timing Advance adjustment delay and accuracy requirement defined in clause 7.3.

##### A.6.4.3.1.2	Test Parameters

Supported test configurations are shown in table A.6.4.3.1.2-1. Both timing advance adjustment delay and accuracy are tested by using the parameters in table A.6.4.3.1.2-2, A.6.4.3.1.2-3 and A.6.4.3.1.2-4.

In all test cases, single cell is used. Each test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time period, timing advance commands are sent to the UE and Sounding Reference Signals (SRS), as specified in table A.6.4.3.1.2-3, are sent from the UE and received by the test equipment. By measuring the reception of the SRS, the transmit timing, and hence the timing advance adjustment accuracy, can be measured.

During time period T1, the test equipment shall send one message with a Timing Advance Command MAC Control Element, as specified in Clause 6.1.3.4 in TS 38.321 [7]. The Timing Advance Command value shall be set to 31, which according to Clause 4.2 in TS 38.213 [3] results in zero adjustment of the Timing Advance. In this way, a reference value for the timing advance used by the UE is established.

During time period T2, the test equipment shall send a sequence of messages with Timing Advance Command MAC Control Elements, with Timing Advance Command value specified in table A.6.4.3.1.2-2. This value shall result in changes of the timing advance used by the UE, and the accuracy of the change shall then be measured, using the SRS sent from the UE.

As specified in Clause 7.3.2.1, the UE adjusts its uplink timing at slot n+k+1 for a timing advance command received in slot n. This delay must be taken into account when measuring the timing advance adjustment accuracy, via the SRS sent from the UE.

The UE Time Alignment Timer, described in Clause 5.2 in TS 38.321 [7], shall be configured so that it does not expire in the duration of the test.

Table A.6.4.3.1.2-1: Timing advance supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.4.3.1.2-2: General test parameters for timing advance

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF channel number |  | 1 |  |
| Initial DL BWP |  | DLBWP.0.1 | As specified in Table A.3.9.2.1-1 |
| Dedicated DL BWP |  | DLBWP.1.1 | As specified in Table A.3.9.2.2-1 |
| Initial UL BWP |  | ULBWP.0.1 | As specified in Table A.3.9.3.1-1 |
| Dedicated UL BWP |  | ULBWP.1.1 | As specified in Table A.3.9.3.2-1 |
| Timing Advance Command (TA) value during T1 |  | 31 | NTA_new = NTA_old for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  | 39 | For 15 kHz SCS NTA_new = NTA_old + 8192*Tc For 30 kHz SCS NTA_new = NTA_old + 4096*Tc (based on equation in clause 4.2 of TS 38.213 [3]) |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.6.4.3.1.2-3: Cell specific test parameters for timing advance

| Parameter |  |  | Unit | Test1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| Duplex mode |  | Config 1 |  | FDD |  |
|  |  | Config 2,3 |  | TDD |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |
| DRx Cycle |  |  | ms | Not Applicable |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.1.1 FDD |  |
|  |  | Config 2 |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |
| SMTC configuration |  | Config 1,2 |  | SMTC.1 FR1 |  |
|  |  | Config 3 |  | SMTC.2 FR1 |  |
| SSB configuration |  | Config 1,2 |  | SSB.1 FR1 |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |
|  |  | Config 3 |  | 30 kHz |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |
|  |  | Config 3 |  | 30 kHz |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |
|  | Config 3 |  |  | -95 |  |
|  |  |  | dB | 3 |  |
|  |  |  | dB | 3 |  |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -67.57 |  |
|  | Config 3 |  | dBm/ 38.16MHz | -62.58 |  |
| Propagation condition |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.6.4.3.1.2-4: Sounding Reference Symbol Configuration for timing advance

| Field |  | Value | Comment |
| --- | --- | --- | --- |
| c-SRS | Config 1,2 | 12 | Frequency hopping is disabled |
|  | Config 3 | 24 |  |
| b-SRS |  | 0 |  |
| b-hop |  | 0 |  |
| freqDomainPosition |  | 0 | Frequency domain position of SRS |
| freqDomainShift |  | 0 |  |
| groupOrSequenceHopping |  | neither | No group or sequence hopping |
| SRS-PeriodicityAndOffset |  | sl5=2 for SCS 15kHz sl5=4 for SCS 30kHz | Once every 5 slots |
| pathlossReferenceRS |  | ssb-Index=0 | SSB #0 is used for SRS path loss estimation |
| usage |  | Codebook | Codebook based UL transmission |
| startPosition |  | 0 | resourceMapping setting. SRS on last symbol of slot, and 1symbols for SRS without repetition. |
| nrofSymbols |  | n1 |  |
| repetitionFactor |  | n1 |  |
| combOffset-n2 |  | 0 | transmissionComb setting |
| cyclicShift-n2 |  | 0 |  |
| nrofSRS-Ports |  | port1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |  |

##### A.6.4.3.1.3	Test Requirements

The UE shall apply the signalled Timing Advance value to the transmission timing at the designated activation time i.e. k+1 slots after the reception of the timing advance command, where k=5.

The Timing Advance adjustment accuracy shall be within the limits specified in clause 7.3.2.2.

The rate of correct Timing Advance adjustments observed during repeated tests shall be at least 90%.

## A.6.5	Signalling characteristics

### A.6.5.1	Radio link Monitoring

In the following clause, any uplink signal transmitted by the UE is used for detecting the In-/Out-of-Sync state of the UE. In terms of measurement, the uplink signal is verified on the basis of the UE output power:

For intra-band contiguous carrier aggregation, transmit OFF power is measured as the mean power per component carrier.

For UE with multiple transmit antennas, transmit OFF power is measured as the mean power at each transmit connector.

-	UE output power higher than Transmit OFF power -50 dBm (as defined in TS 38.101-1 [18]) means uplink signal

-	UE output power equal to or less than Transmit OFF power -50 dBm (as defined in TS 38.101-1 [18]) means no uplink signal.

#### A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode

##### A.6.5.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the FR1 radio link monitoring requirements in clause 8.1.

In the test, UE is configured to perform RLM on SSB, with detectionResource included in RadioLinkMonitoringRS set to SSB#0, and purpose set to ‘rlf’. Supported test configurations are shown in table A.6.5.1.1.1-1. The test parameters are given in Tables A.6.5.1.1.1-2, A.6.5.1.1.1-3, and A.6.5.1.1.1-4 below. There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.6.5.1.1.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. The UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40ms) in test 1.

Table A.6.5.1.1.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.1.1.1-2: General test parameters for FR1 out-of-sync testing in non-DRX mode

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2, 3 |  | TDD |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |
|  |  | Config 2 |  | 10: NRB,c = 52 |
|  |  | Config 3 |  | 40: NRB,c = 106 |
| DL initial BWP configuration |  | Config 1, 2, 3 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1, 2, 3 |  | DLBWP.1.1 |
| UL initial BWP configuration |  | Config 1, 2, 3 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  | Config 1, 2, 3 |  | ULBWP.1.1 |
| TDD Configuration |  | Config 1 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.1.3 FDD |
|  |  | Config 2 |  | CCR.1.3 TDD |
|  |  | Config 3 |  | CCR.2.2 TDD |
| SSB Configuration |  | Config 1 |  | SSB.1 FR1 |
|  |  | Config 2 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration |  | Config 1, 2 |  | SMTC.1 |
|  |  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2 |  | 15 kHz |
|  |  | Config 3 |  | 30 kHz |
| PRACH Configuration |  | Config 1, 2 |  | Table A.3.8.2.1-1 |
|  |  | Config 3 |  | Table A.3.8.2.1-1 |
| SSB index assigned as RLM RS |  |  |  | 0 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX |  |  |  | OFF |
| Gap pattern ID |  |  |  | gp0 |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 0 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration for CSI reporting |  | Config 1 |  | CSI-RS.1.1 FDD |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |
| CSI-RS for tracking |  | Config 1 |  | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 0.48 |
| T3 |  |  | s | 0.48 |
| D1 |  |  | s | 0.44 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

Table A.6.5.1.1.1-3: Cell specific test parameters for FR1 (Cell 1) for out-of-sync radio link monitoring tests in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |
| SNR on RLM-RS | Config 1 | dB | 1 | -7 | -15 |
|  | Config 2 |  | 1 | -7 | -15 |
|  | Config 3 |  | 1 | -7 | -15 |
|  | Config 1 | dBm/15kHz | -98 |  |  |
|  | Config 2 |  | -98 |  |  |
|  | Config 3 |  | -98 |  |  |
|  | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 |  | -98 |  |  |
|  | Config 3 |  | -95 |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in Figure A.6.5.1.1.1-1. Note 5: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is A.3.6. |  |  |  |  |  |

Table A.6.5.1.1.1-4: Measurement gap configuration for out-of-sync tests in non-DRX mode

| Field | Test 1 |
| --- | --- |
|  | Value |
| gapOffset | 0 |
| Note: Ensure that RLM RS is partially overlapped with measurement gap |  |

Figure A.6.5.1.1.1-1: SNR variation for out-of-sync testing

##### A.6.5.1.1.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The UE shall stop transmitting uplink signal no later than time point C (D1 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode

##### A.6.5.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the FR1 radio link monitoring requirements in clause 8.1.

In the test, UE is configured to perform RLM on SSB, with detectionResource included in RadioLinkMonitoringRS set to SSB#0, and purpose set to ‘rlf’. Supported test configurations are shown in table A.6.5.1.2.1-1. The test parameters are given in Tables A.6.5.1.2.1-2, and A.6.5.1.2.1-3 below. There is one cell (Cell 1), which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.6.5.1.2.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms.

Table A.6.5.1.2.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.1.2.1-2: General test parameters for FR1 in-sync testing in non-DRX mode

| Parameter |  |  |  | Unit | Value |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 |
| Active PCell |  |  |  |  | Cell 1 |
| RF Channel Number |  |  |  |  | 1 |
| Duplex mode |  |  | Config 1 |  | FDD |
|  |  |  | Config 2, 3 |  | TDD |
| BWchannel |  |  | Config 1 | MHz | 10: NRB,c = 52 |
|  |  |  | Config 2 |  | 10: NRB,c = 52 |
|  |  |  | Config 3 |  | 40: NRB,c = 106 |
| DL initial BWP configuration |  |  | Config 1, 2, 3 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  |  | Config 1, 2, 3 |  | DLBWP.1.1 |
| UL initial BWP configuration |  |  | Config 1, 2, 3 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  |  | Config 1, 2, 3 |  | ULBWP.1.1 |
| TDD Configuration |  |  | Config 1 |  | Not Applicable |
|  |  |  | Config 2 |  | TDDConf.1.1 |
|  |  |  | Config 3 |  | TDDConf.2.1 |
| RMSI CORESET Reference Channel |  |  | Config 1 |  | CR.1.1 FDD |
|  |  |  | Config 2 |  | CR.1.1 TDD |
|  |  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  |  | Config 1 |  | CCR.1.1 FDD |
|  |  |  | Config 2 |  | CCR.1.1 TDD |
|  |  |  | Config 3 |  | CCR.2.1 TDD |
| SSB Configuration |  |  | Config 1 |  | SSB.1 FR1 |
|  |  |  | Config 2 |  | SSB.1 FR1 |
|  |  |  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration |  |  | Config 1, 2 |  | SMTC.1 |
|  |  |  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1, 2 |  | 15 kHz |
|  |  |  | Config 3 |  | 30 kHz |
| PRACH Configuration |  |  | Config 1, 2 |  | Table A.3.8.2.1-1 |
|  |  |  | Config 3 |  | Table A.3.8.2.1-1 |
| SSB index assigned as RLM RS |  |  |  |  | 0 |
| OCNG parameters |  |  |  |  | OP.1 |
| CP length |  |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 2x2 Low |
| In sync transmission parameters | DCI format |  |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  |  | 2 |
|  | Aggregation level |  |  | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB | 0 |
|  | DMRS precoder granularity |  |  |  | REG bundle size |
|  | REG bundle size |  |  |  | 6 |
| Out of sync transmission parameters | DCI format |  |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  |  | 2 |
|  | Aggregation level |  |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB | 4 |
|  | DMRS precoder granularity |  |  |  | REG bundle size |
|  | REG bundle size |  |  |  | 6 |
| DRX |  |  |  |  | OFF |
| Gap pattern ID |  |  |  |  | N.A. |
| Layer 3 filtering |  |  |  |  | Enabled |
| T310 timer |  |  |  | ms | 1000 |
| T311 timer |  |  |  | ms | 1000 |
| N310 |  |  |  |  | 1 |
| N311 |  |  |  |  | 1 |
| CSI-RS configuration for CSI reporting |  | Config 1 |  |  | CSI-RS.1.1 FDD |
|  |  | Config 2 |  |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  |  | CSI-RS.2.1 TDD |
| CSI-RS for tracking |  | Config 1, 4 |  |  | TRS.1.1 FDD |
|  |  | Config 2, 5 |  |  | TRS.1.1 TDD |
|  |  | Config 3, 6 |  |  | TRS.1.2 TDD |
| T1 |  |  |  | s | 0.2 |
| T2 |  |  |  | s | 0.2 |
| T3 |  |  |  | s | 0.24 |
| T4 |  |  |  | s | 0.2 |
| T5 |  |  |  | s | 0.88 |
| D1 |  |  |  | s | 0.84 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

Table A.6.5.1.2.1-3: Cell specific test parameters for FR1 (Cell 1) for in-sync radio link monitoring tests in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR on RLM-RS | Config 1 | dB | 1 | -7 | -15 | -4.5 | 1 |
|  | Config 2 |  | 1 | -7 | -15 | -4.5 | 1 |
|  | Config 3 |  | 1 | -7 | -15 | -4.5 | 1 |
|  | Config 1 | dBm/15 kHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
|  | Config 1 | dBm/SCS | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -95 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in Figure A.6.5.1.2.1-1. Note 5: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in clause A.3.6. |  |  |  |  |  |  |  |

Table A.6.5.1.2.1-4: Void

Figure A.6.5.1.2.1-1: SNR variation for in-sync testing

##### A.6.5.1.2.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode

##### A.6.5.1.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the FR1 radio link monitoring requirements in clause 8.1.

In the test, UE is configured to perform RLM on SSB, with detectionResource included in RadioLinkMonitoringRS set to SSB#0, and purpose set to ‘rlf’. Supported test configurations are shown in table A.6.5.1.3.1-1. The test parameters are given in Tables A.6.5.1.3.1-2, and A.6.5.1.3.1-3. There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.6.5.1.3.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CSI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.6.5.1.3.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.1.3.1-2: General test parameters for FR1 out-of-sync testing in DRX mode

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2, 3 |  | TDD |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |
|  |  | Config 2 |  | 10: NRB,c = 52 |
|  |  | Config 3 |  | 40: NRB,c = 106 |
| DL initial BWP configuration |  | Config 1, 2, 3 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1, 2, 3 |  | DLBWP.1.1 |
| UL initial BWP configuration |  | Config 1, 2, 3 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  | Config 1, 2, 3 |  | ULBWP.1.1 |
| TDD Configuration |  | Config 1 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.1.3 FDD |
|  |  | Config 2 |  | CCR.1.3 TDD |
|  |  | Config 3 |  | CCR.2.2 TDD |
| SSB Configuration |  | Config 1 |  | SSB.1 FR1 |
|  |  | Config 2 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration |  | Config 1, 2 |  | SMTC.1 |
|  |  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2 |  | 15 kHz |
|  |  | Config 3 |  | 30 kHz |
| PRACH Configuration |  | Config 1, 2 |  | Table A.3.8.2.1-1 |
|  |  | Config 3 |  | Table A.3.8.2.1-1 |
| SSB index assigned as RLM RS |  |  |  | 0 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX Configuration |  |  |  | DRX.3 |
| Gap pattern ID |  |  |  | N.A. |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 0 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration for CSI reporting |  | Config 1 |  | CSI-RS.1.1 FDD |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |
| CSI-RS for tracking |  | Config 1 |  | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 0.68 |
| T3 |  |  | s | 0.68 |
| D1 |  |  | s | 0.64 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

Table A.6.5.1.3.1-3: Cell specific test parameters for FR1 (Cell 1) for out-of-sync radio link monitoring tests in DRX mode

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |
| SNR on RLM-RS | Config 1 | dB | 1 | -7 | -15 |
|  | Config 2 |  | 1 | -7 | -15 |
|  | Config 3 |  | 1 | -7 | -15 |
|  | Config 1 | dBm/15kHz | -98 |  |  |
|  | Config 2 |  | -98 |  |  |
|  | Config 3 |  | -98 |  |  |
|  | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 |  | -98 |  |  |
|  | Config 3 |  | -95 |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in Figure A.6.5.1.3.1-1. Note 5: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is A.3.6. |  |  |  |  |  |

Table A.6.5.1.3.1-4: Void

Table A.6.5.1.3.1-5: Void

Table A.6.5.1.3.1-6: Void

Figure A.6.5.1.3.1-1: SNR variation for out-of-sync testing

##### A.6.5.1.3.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The UE shall stop transmitting uplink signal no later than time point C (D1 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode

##### A.6.5.1.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the FR1 radio link monitoring requirements in clause 8.1.

In the test, UE is configured to perform RLM on SSB, with detectionResource included in RadioLinkMonitoringRS set to SSB#0, and purpose set to ‘rlf’. Supported test configurations are shown in table A.6.5.1.4.1-1. The test parameters are given in Tables A.6.5.1.4.1-2, and A.6.5.1.4.1-3. There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.6.5.1.4.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CSI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.6.5.1.4.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.1.4.1-2: General test parameters for FR1 in-sync testing in DRX mode

| Parameter |  |  |  | Unit | Value |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 |
| Active PCell |  |  |  |  | Cell 1 |
| RF Channel Number |  |  |  |  | 1 |
| Duplex mode |  |  | Config 1 |  | FDD |
|  |  |  | Config 2, 3 |  | TDD |
| BWchannel |  |  | Config 1 | MHz | 10: NRB,c = 52 |
|  |  |  | Config 2 |  | 10: NRB,c = 52 |
|  |  |  | Config 3 |  | 40: NRB,c = 106 |
| DL initial BWP configuration |  |  | Config 1, 2, 3 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  |  | Config 1, 2, 3 |  | DLBWP.1.1 |
| UL initial BWP configuration |  |  | Config 1, 2, 3 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  |  | Config 1, 2, 3 |  | ULBWP.1.1 |
| TDD Configuration |  |  | Config 1 |  | Not Applicable |
|  |  |  | Config 2 |  | TDDConf.1.1 |
|  |  |  | Config 3 |  | TDDConf.2.1 |
| RMSI CORESET Reference Channel |  |  | Config 1 |  | CR.1.1 FDD |
|  |  |  | Config 2 |  | CR.1.1 TDD |
|  |  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  |  | Config 1 |  | CCR.1.1 FDD |
|  |  |  | Config 2 |  | CCR.1.1 TDD |
|  |  |  | Config 3 |  | CCR.2.1 TDD |
| SSB Configuration |  |  | Config 1 |  | SSB.1 FR1 |
|  |  |  | Config 2 |  | SSB.1 FR1 |
|  |  |  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration |  |  | Config 1, 2 |  | SMTC.1 |
|  |  |  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1, 2 |  | 15 kHz |
|  |  |  | Config 3 |  | 30 kHz |
| PRACH Configuration |  |  | Config 1, 2 |  | Table A.3.8.2.1-1 |
|  |  |  | Config 3 |  | Table A.3.8.2.1-1 |
| SSB index assigned as RLM RS |  |  |  |  | 0 |
| OCNG parameters |  |  |  |  | OP.1 |
| CP length |  |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 2x2 Low |
| In sync transmission parameters | DCI format |  |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  |  | 2 |
|  | Aggregation level |  |  | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB | 0 |
|  | DMRS precoder granularity |  |  |  | REG bundle size |
|  | REG bundle size |  |  |  | 6 |
| Out of sync transmission parameters | DCI format |  |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  |  | 2 |
|  | Aggregation level |  |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB | 4 |
|  | DMRS precoder granularity |  |  |  | REG bundle size |
|  | REG bundle size |  |  |  | 6 |
| DRX Configuration |  |  |  |  | DRX.3 |
| Gap pattern ID |  |  |  |  | N.A. |
| Layer 3 filtering |  |  |  |  | Enabled |
| T310 timer |  |  |  | ms | 2000 |
| T311 timer |  |  |  | ms | 1000 |
| N310 |  |  |  |  | 1 |
| N311 |  |  |  |  | 1 |
| CSI-RS configuration for CSI reporting |  | Config 1 |  |  | CSI-RS.1.1 FDD |
|  |  | Config 2 |  |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  |  | CSI-RS.2.1 TDD |
| CSI-RS for tracking |  | Config 1 |  |  | TRS.1.1 FDD |
|  |  | Config 2 |  |  | TRS.1.1 TDD |
|  |  | Config 3 |  |  | TRS.1.2 TDD |
| T1 |  |  |  | s | 0.2 |
| T2 |  |  |  | s | 0.2 |
| T3 |  |  |  | s | 0.64 |
| T4 |  |  |  | s | 0.2 |
| T5 |  |  |  | s | 0.88 |
| D1 |  |  |  | s | 0.84 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

Table A.6.5.1.4.1-3: Cell specific test parameters for FR1 (Cell 1) for in-sync radio link monitoring tests in DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR on RLM-RS | Config 1 | dB | 1 | -7 | -15 | -4.5 | 1 |
|  | Config 2 |  | 1 | -7 | -15 | -4.5 | 1 |
|  | Config 3 |  | 1 | -7 | -15 | -4.5 | 1 |
|  | Config 1 | dBm/15 kHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
|  | Config 1 | dBm/SCS | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -95 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in Figure A.6.5.1.4.1-1. Note 5: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in clause A.3.6. |  |  |  |  |  |  |  |

Table A.6.5.1.4.1-4: Void

Table A.6.5.1.4.1-5: Void

Figure A.6.5.1.4.1-1: SNR variation for in-sync testing.

##### A.6.5.1.4.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode

##### A.6.5.1.5.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when no DRX is used. This test will partly verify the FR1 PCell CSI-RS Out-of-sync radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.6.5.1.5.1-1, A.6.5.1.5.1-2, A.6.5.1.5.1-3, and A.6.5.1.5.1-3A below. There is one cell, cell 1 which is the PCell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.6.5.1.5.1-1 shows the variation of the downlink SNR in the PCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting of 5ms. In the test, DRX configuration is not enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms) in test. In the test, SSB0 is configured as the BFD-RS.

Table A.6.5.1.5.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 3 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.1.5.1-2: General test parameters for FR1 PCell for CSI-RS out-of-sync testing in non-DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode | Config 1 |  | FDD |
|  | Config 2, 3 |  | TDD |
| TDD Configuration | Config 1 |  | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.2.1 |
| DL initial BWP configuration | Config 1, 2, 3 |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1, 2, 3 |  | DLBWP.1.1 |
| UL initial BWP configuration | Config 1, 2, 3 |  | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1, 2, 3 |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.1.3 FDD |
|  | Config 2 |  | CCR.1.3 TDD |
|  | Config 3 |  | CCR.2.2 TDD |
| SSB Configuration | Config 1 |  | SSB.1 FR1 |
|  | Config 2 |  | SSB.1 FR1 |
|  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration | Config 1, 2 |  | SMTC.1 |
|  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1, 2 |  | 15 kHz |
|  | Config 3 |  | 30 kHz |
| TRS configuration | Config 1 |  | TRS.1.1 FDD |
|  | Config 2 |  | TRS.1.1 TDD |
|  | Config 3 |  | TRS.1.2 TDD |
| CSI-RS for RLM | Config 1 |  | Resource #4 in TRS.1.1 FDD |
|  | Config 2 |  | Resource #4 in TRS.1.1 TDD |
|  | Config 3 |  | Resource #4 in TRS.1.2 TDD |
| TCI configuration for PDCCH/PDSCH |  |  | TCI.State. 2 |
| OCNG parameters |  |  | OP.1 |
| CP length |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | OFF |
| Gap pattern ID |  |  | gp0 |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 0 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS configuration for CSI reporting | Config 1 |  | CSI-RS.1.1 FDD |
|  | Config 2 |  | CSI-RS.1.1 TDD |
|  | Config 3 |  | CSI-RS.2.1 TDD |
| T1 |  | s | 0.2 |
| T2 |  | s | 0.48 |
| T3 |  | s | 0.48 |
| D1 |  | s | 0.44 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |

Table A.6.5.1.5.1-3: Cell specific test parameters for FR1 for CSI-RS out-of-sync radio link monitoring in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| EPRE ratio of PDCCH DMRS to SSSPDCCH_beta |  | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRSPDCCH_DMRS_beta |  | dB |  |  |  |
| EPRE ratio of PBCH DMRS to SSSPBCH_beta |  | dB | 0 |  |  |
| EPRE ratio of PBCH to PBCH DMRSPSS_beta |  | dB |  |  |  |
| EPRE ratio of PSS to SSSSSS_beta |  | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS PDSCH_beta |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |
| SNR on RLM-RS | Config 1 | dB | 1 | -7 | -15 |
|  | Config 2 |  | 1 | -7 | -15 |
|  | Config 3 |  | 1 | -7 | -15 |
|  | Config 1 | dBm/15kHz | -98 |  |  |
|  | Config 2 |  | -98 |  |  |
|  | Config 3 |  | -98 |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.6.5.1.5.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is [A.3.6]. |  |  |  |  |  |

Table A.6.5.1.5.1-3A: Measurement gap configuration for FR1 CSI-RS out-of-sync radio link monitoring in non-DRX mode

| Field | Test 1 |
| --- | --- |
|  | Value |
| gapOffset | 0 |
| Note 1: Void |  |

Table A.6.5.1.5.1-4: Void

Figure A.6.5.1.5.1-1: SNR variation for CSI-RS out-of-sync testing

##### A.6.5.1.5.2	Test Requirements

The UE behaviour during time durations T1, T2, and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

The UE shall stop transmitting uplink signal in Cell 1 no later than time point C (D1 ms after the start of the time duration T3) on the PCell.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode

##### A.6.5.1.6.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the in sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when no DRX is used. This test will partly verify the FR1 PCell CSI-RS In-sync radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.6.5.1.6.1-1, A.6.5.1.6.1-2, and A.6.5.1.6.1-3 below. There is one cells, cell 1which is the PCell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.6.5.1.6.1-1 shows the variation of the downlink SNR in the PCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5ms. In the test, DRX configuration is not enabled. In the test, SSB0 is configured as the BFD-RS.

Table A.6.5.1.6.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 3 | TDD duplex mode, 30kHz SSB SCS, 40 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.1.6.1-2: General test parameters for FR1 PCell for CSI-RS in-sync testing in non-DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode | Config 1 |  | FDD |
|  | Config 2, 3 |  | TDD |
| TDD Configuration | Config 1 |  | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.2.1 |
| DL initial BWP configuration | Config 1, 2, 3 |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1, 2, 3 |  | DLBWP.1.1 |
| UL initial BWP configuration | Config 1, 2, 3 |  | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1, 2, 3 |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.1.1 FDD |
|  | Config 2 |  | CCR.1.1 TDD |
|  | Config 3 |  | CCR.2.1 TDD |
| SSB Configuration | Config 1 |  | SSB.1 FR1 |
|  | Config 2 |  | SSB.1 FR1 |
|  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration | Config 1, 2 |  | SMTC.1 |
|  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1, 2 |  | 15 kHz |
|  | Config 3 |  | 30 kHz |
| TRS configuration | Config 1 |  | TRS.1.1 FDD |
|  | Config 2 |  | TRS.1.1 TDD |
|  | Config 3 |  | TRS.1.2 TDD |
| CSI-RS for RLM | Config 1 |  | Resource #4 in TRS.1.1 FDD |
|  | Config 2 |  | Resource #4 in TRS.1.1 TDD |
|  | Config 3 |  | Resource #4 in TRS.1.2 TDD |
| TCI configuration for PDCCH/PDSCH |  |  | TCI.State. 2 |
| OCNG parameters |  |  | OP.1 |
| CP length |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| In sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 0 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | OFF |
| Gap pattern ID |  |  | N.A. |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 1000 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS configuration for CSI reporting | Config 1 |  | CSI-RS.1.1 FDD |
|  | Config 2 |  | CSI-RS.1.1 TDD |
|  | Config 3 |  | CSI-RS.2.1 TDD |
| T1 |  | s | 0.2 |
| T2 |  | s | 0.2 |
| T3 |  | s | 0.44 |
| T4 |  | s | 0.2 |
| T5 |  | s | 0.88 |
| T6 |  | S | 0.84 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |

Table A.6.5.1.6.1-3: Cell specific test parameters for FR1 for CSI-RS in-sync radio link monitoring in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSSPDCCH_beta |  | dB | 4 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRSPDCCH_DMRS_beta |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSSPBCH_beta |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRSPSS_beta |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSSSSS_beta |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS PDSCH_beta |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR on RLM-RS | Config 1 | dB | 1 | -7 | -15 | -4.5 | 1 |
|  | Config 2 |  | 1 | -7 | -15 | -4.5 | 1 |
|  | Config 3 |  | 1 | -7 | -15 | -4.5 | 1 |
|  | Config 1 | dBm/15kHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.6.5.1.6.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is specified in clause A.3.6.1.1. |  |  |  |  |  |  |  |

Table A.6.5.1.6.1-4: Void

Figure A.6.5.1.6.1-1: SNR variation for CSI-RS in-sync testing

##### A.6.5.1.6.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (T6 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting on the PCell.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode

##### A.6.5.1.7.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when DRX is used. This test will partly verify the FR1 PCell CSI-RS Out-of-sync radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.6.5.1.7.1-1, A.6.5.1.7.1-2, and A.6.5.1.7.1-3 below. There is one cell, cell 1 is the PCell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.6.5.1.7.1-1 shows the variation of the downlink SNR in the PCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5ms. In the test, DRX configuration is enabled in PCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test. In the test, SSB0 is configured as the BFD-RS.

Table A.6.5.1.7.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 3 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.1.7.1-2: General test parameters for FR1 PCell for CSI-RS out-of-sync testing in DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode | Config 1 |  | FDD |
|  | Config 2, 3 |  | TDD |
| TDD Configuration | Config 1 |  | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.2.1 |
| DL initial BWP configuration | Config 1, 2, 3 |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1, 2, 3 |  | DLBWP.1.1 |
| UL initial BWP configuration | Config 1, 2, 3 |  | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1, 2, 3 |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.1.3 FDD |
|  | Config 2 |  | CCR.1.3 TDD |
|  | Config 3 |  | CCR.2.2 TDD |
| SSB Configuration | Config 1 |  | SSB.1 FR1 |
|  | Config 2 |  | SSB.1 FR1 |
|  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration | Config 1, 2 |  | SMTC.1 |
|  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1, 2 |  | 15 kHz |
|  | Config 3 |  | 30 kHz |
| TRS configuration | Config 1 |  | TRS.1.1 FDD |
|  | Config 2 |  | TRS.1.1 TDD |
|  | Config 3 |  | TRS.1.2 TDD |
| CSI-RS for RLM | Config 1 |  | Resource #4 in TRS.1.1 FDD |
|  | Config 2 |  | Resource #4 in TRS.1.1 TDD |
|  | Config 3 |  | Resource #4 in TRS.1.2 TDD |
| TCI configuration for PDCCH/PDSCH |  |  | TCI.State. 2 |
| OCNG parameters |  |  | OP.1 |
| CP length |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | DRX.3 |
| Gap pattern ID |  |  | N.A. |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 0 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS configuration for CSI reporting | Config 1 |  | CSI-RS.1.1 FDD |
|  | Config 2 |  | CSI-RS.1.1 TDD |
|  | Config 3 |  | CSI-RS.2.1 TDD |
| T1 |  | s | 0.2 |
| T2 |  | s | 1.28 |
| T3 |  | s | 1.28 |
| D1 |  | s | 1.24 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |

Table A.6.5.1.7.1-3: Cell specific test parameters for FR1 for CSI-RS out-of-sync radio link monitoring in DRX mode

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| EPRE ratio of PDCCH DMRS to SSSPDCCH_beta |  | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRSPDCCH_DMRS_beta |  | dB | 4 |  |  |
| EPRE ratio of PBCH DMRS to SSSPBCH_beta |  | dB | 0 |  |  |
| EPRE ratio of PBCH to PBCH DMRSPSS_beta |  | dB |  |  |  |
| EPRE ratio of PSS to SSSSSS_beta |  | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS PDSCH_beta |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |
| SNR on RLM-RS | Config 1 | dB | 1 | -7 | -15 |
|  | Config 2 |  | 1 | -7 | -15 |
|  | Config 3 |  | 1 | -7 | -15 |
|  | Config 1 | dBm/15kHz | -98 |  |  |
|  | Config 2 |  | -98 |  |  |
|  | Config 3 |  | -98 |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.6.5.1.7.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is specified in clause A.3.6.1.1. |  |  |  |  |  |

Table A.6.5.1.7.1-4: Void

Table A.6.5.1.7.1-5: Void

Table A.6.5.1.7.1-6: Void

Figure A.6.5.1.7.1-1: SNR variation for CSI-RS out-of-sync testing

##### A.6.5.1.7.2	Test Requirements

The UE behaviour during time durations T1, T2, and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 (PCell) at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

The UE shall stop transmitting uplink signal in Cell 1 (PCell) no later than time point C (D1 ms after the start of the time duration T3) on the PCell.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode

##### A.6.5.1.8.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the in sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when DRX is used. This test will partly verify the FR1 PCell CSI-RS In-sync radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.6.5.1.8.1-1, A.6.5.1.81-2, A.6.5.1.8.1-3 and A.6.5.1.8.1-3A below. There is one cells, cell 1which is the PCell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.6.5.1.8.1-1 shows the variation of the downlink SNR in the PCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5ms. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms) in test. In the test, SSB0 is configured as the BFD-RS.

Table A.6.5.1.8.1-1: Supported test configurations for FR1 PSCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 3 | TDD duplex mode, 30kHz SSB SCS, 40 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.1.8.1-2: General test parameters for FR1 PCell for CSI-RS in-sync testing in non-DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode | Config 1 |  | FDD |
|  | Config 2, 3 |  | TDD |
| TDD Configuration | Config 1 |  | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.2.1 |
| DL initial BWP configuration | Config 1, 2, 3 |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1, 2, 3 |  | DLBWP.1.1 |
| UL initial BWP configuration | Config 1, 2, 3 |  | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1, 2, 3 |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.1.1 FDD |
|  | Config 2 |  | CCR.1.1 TDD |
|  | Config 3 |  | CCR.2.1 TDD |
| SSB Configuration | Config 1 |  | SSB.1 FR1 |
|  | Config 2 |  | SSB.1 FR1 |
|  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration | Config 1, 2 |  | SMTC.1 |
|  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1, 2 |  | 15 kHz |
|  | Config 3 |  | 30 kHz |
| TRS configuration | Config 1 |  | TRS.1.1 FDD |
|  | Config 2 |  | TRS.1.1 TDD |
|  | Config 3 |  | TRS.1.2 TDD |
| CSI-RS for RLM | Config 1 |  | Resource #4 in TRS.1.1 FDD |
|  | Config 2 |  | Resource #4 in TRS.1.1 TDD |
|  | Config 3 |  | Resource #4 in TRS.1.2 TDD |
| TCI configuration for PDCCH/PDSCH |  |  | TCI.State. 2 |
| OCNG parameters |  |  | OP.1 |
| CP length |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| In sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 0 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | DRX.3 |
| Gap pattern ID |  |  | gp0 |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 4000 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS configuration for CSI reporting | Config 1 |  | CSI-RS.1.1 FDD |
|  | Config 2 |  | CSI-RS.1.1 TDD |
|  | Config 3 |  | CSI-RS.2.1 TDD |
| T1 |  | s | 0.2 |
| T2 |  | s | 0.2 |
| T3 |  | s | 1.24 |
| T4 |  | s | 0.2 |
| T5 |  | s | 4 |
| T6 |  | s | 3.88 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |

Table A.6.5.1.8.1-3: Cell specific test parameters for FR1 for CSI-RS in-sync radio link monitoring in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSSPDCCH_beta |  | dB | 4 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRSPDCCH_DMRS_beta |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSSPBCH_beta |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRSPSS_beta |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSSSSS_beta |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS PDSCH_beta |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR on RLM-RS | Config 1 | dB | 1 | -7 | -15 | -4.5 | 1 |
|  | Config 2 |  | 1 | -7 | -15 | -4.5 | 1 |
|  | Config 3 |  | 1 | -7 | -15 | -4.5 | 1 |
|  | Config 1 | dBm/15kHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.6.5.1.8.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is specified in clause A.3.6.1.1. |  |  |  |  |  |  |  |

Table A.6.5.1.8.1-3A: Measurement gap configuration for FR1 CSI-RS in-sync radio link monitoring in non-DRX mode

| Field | Test 1 |
| --- | --- |
|  | Value |
| gapOffset | 0 |
| Note 1: Void |  |

Table A.6.5.1.8.1-4: Void

Table A.6.5.1.8.1-5: Void

Table A.6.5.1.8.1-6: Void

Figure A.6.5.1.8.1-1: SNR variation for CSI-RS in-sync testing

##### A.6.5.1.8.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (T6 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting on the PCell.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion

##### A.6.5.1.9.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when DRX is used. This test will partly verify the FR1 PCell CSI-RS Out-of-sync radio link monitoring requirements in clause 8.1.3.4 for UE fulfilling good serving cell quality criterion.

The test parameters are given in Tables A.6.5.1.9.1-1, A.6.5.1.9.1-2, and A.6.5.1.9.1-3 below. There is one cell, cell 1 is the PCell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.6.5.1.9.1-1 shows the variation of the downlink SNR in the PCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5ms. In the test, DRX configuration is enabled in PCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test. In the test, SSB0 is configured as the BFD-RS.

Table A.6.5.1.9.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 3 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.1.9.1-2: General test parameters for FR1 PCell for CSI-RS out-of-sync testing in DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode | Config 1 |  | FDD |
|  | Config 2, 3 |  | TDD |
| TDD Configuration | Config 1 |  | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.2.1 |
| DL initial BWP configuration | Config 1, 2, 3 |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1, 2, 3 |  | DLBWP.1.1 |
| UL initial BWP configuration | Config 1, 2, 3 |  | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1, 2, 3 |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.1.3 FDD |
|  | Config 2 |  | CCR.1.3 TDD |
|  | Config 3 |  | CCR.2.2 TDD |
| SSB Configuration | Config 1 |  | SSB.1 FR1 |
|  | Config 2 |  | SSB.1 FR1 |
|  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration | Config 1, 2 |  | SMTC.1 |
|  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1, 2 |  | 15 kHz |
|  | Config 3 |  | 30 kHz |
| TRS configuration | Config 1 |  | TRS.1.1 FDD |
|  | Config 2 |  | TRS.1.1 TDD |
|  | Config 3 |  | TRS.1.2 TDD |
| CSI-RS for RLM | Config 1 |  | Resource #4 in TRS.1.1 FDD |
|  | Config 2 |  | Resource #4 in TRS.1.1 TDD |
|  | Config 3 |  | Resource #4 in TRS.1.2 TDD |
| TCI configuration for PDCCH/PDSCH |  |  | TCI.State. 2 |
| OCNG parameters |  |  | OP.1 |
| CP length |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | [DRX.X1] |
| Gap pattern ID |  |  | N.A. |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 0 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS configuration for CSI reporting | Config 1 |  | CSI-RS.1.1 FDD |
|  | Config 2 |  | CSI-RS.1.1 TDD |
|  | Config 3 |  | CSI-RS.2.1 TDD |
| T1 |  | s | 0.2 |
| T2 |  | s | 2.48 |
| T3 |  | s | 4.88 |
| D1 |  | s | 4.84 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |

Table A.6.5.1.9.1-3: Cell specific test parameters for FR1 for CSI-RS out-of-sync radio link monitoring in DRX mode

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| EPRE ratio of PDCCH DMRS to SSSPDCCH_beta |  | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRSPDCCH_DMRS_beta |  | dB | 4 |  |  |
| EPRE ratio of PBCH DMRS to SSSPBCH_beta |  | dB | 0 |  |  |
| EPRE ratio of PBCH to PBCH DMRSPSS_beta |  | dB |  |  |  |
| EPRE ratio of PSS to SSSSSS_beta |  | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS PDSCH_beta |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |
| SNR on RLM-RS | Config 1 | dB | 1 | 1 | -15 |
|  | Config 2 |  | 1 | 1 | -15 |
|  | Config 3 |  | 1 | 1 | -15 |
|  | Config 1 | dBm/15kHz | -98 |  |  |
|  | Config 2 |  | -98 |  |  |
|  | Config 3 |  | -98 |  |  |
| goodServingCellEvaluationRLM |  |  | configured |  |  |
| offset in goodServingCellEvaluationRLM |  | dB | Not configured |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.6.5.1.7.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is specified in clause A.3.6.1.1. |  |  |  |  |  |

Figure A.6.5.1.9.1-1: SNR variation for CSI-RS out-of-sync testing

##### A.6.5.1.9.2	Test Requirements

The UE behaviour during time durations T1, T2, and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 (PCell) at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

The UE shall stop transmitting uplink signal in Cell 1 (PCell) no later than time point C (D1 ms after the start of the time duration T3) on the PCell.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.6.5.2	Interruption

#### A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1

##### A.6.5.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE missed ACK/NACK rate does not exceed the limits at NR PSCell interruptions during the measurement on the deactivated NR SCC. This test will verify the missed ACK/NACK rate for PCell in standalone NR specified in clause 8.2.2.2. Supported test configurations for NR PCell are shown in table A.6.5.2.1.1-1. Supported test configurations for NR SCell are shown in table A.6.5.2.1.1-1A. Test configuration for NR PCell and test configuration for NR SCell are chosen independently.

The general test parameters and NR cell specific test parameters are given in Table A.6.5.2.1.1-2, A.6.5.2.1.1-3 and A.6.5.2.1.1-4 below. In the test there are two cells: Cell1 and Cell2. Cell1 is PCell, Cell2 is an NR deactivated SCell. Cell1 shall be configured as PCell and Cell2 shall be configured as SCell.

The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, the UE is connected to Cell1 and Cell2 and the RRC message including measCycleSCell or allowInterruptions for the deactivated NR SCells is received at the UE antenna connector. During T1, PCell is continuously scheduled in DL.

Table A.6.5.2.1.1-1: Interruptions during measurements on deactivated NR SCC supported test configurations for NR PCell

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table A.6.5.2.1.1-1A: Interruptions during measurements on deactivated NR SCC supported test configurations for NR SCell

| ConfigSCell | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table A.6.5.2.1.1-2: General test parameters for interruptions during measurements on deactivated NR SCC in standalone NR

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | Two NR RF channels |
| Active PCell |  | Cell1 | PCell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell2 | Deactivated SCell on NR RF channel number 2. |
| CP length |  | Normal | Applicable to Cell1 and Cell2 |
| DRX |  | OFF |  |
| Measurement gap pattern Id |  | OFF |  |
| SCell measurement cycle (measCycleSCell) | ms | 640 |  |
| T1 | s | 10 |  |

Table A.6.5.2.1.1-3: NR cell specific test parameters for NR PCell for interruptions during measurements on deactivated NR SCC in standalone NR

| Parameter |  | Unit | Cell1 |
| --- | --- | --- | --- |
| Frequency Range |  |  | FR1 |
| Duplex mode | Config 1 |  | FDD |
|  | Config 2,3 |  | TDD |
| TDD configuration | Config 1 |  | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 |
|  | Confiq 3 |  | TDDConf.2.1 |
| BWchannel | Config 1,2 |  | Note 9 |
|  | Config 3 |  | Note 9 |
| BWoccupied | Config 1,2 | RB | 52 Note 7 |
|  | Config 3 |  | 106 Note 8 |
| Initial DL BWP Configuration | Config 1,2,3 |  | DLBWP.0.1 |
| Dedicated DL BWP Configuration | Config 1,2,3 |  | DLBWP.1.1 |
| Initial UL BWP Configuration | Config 1,2,3 |  | ULBWP.0.1 |
| Dedicated UL BWP Configuration | Config 1,2,3 |  | ULBWP.1.1 |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |
|  | Config 2 |  | SR.1.2 TDD |
|  | Config 3 |  | SR.2.1 TDD |
| CSI-RS for tracking | Config 1 |  | TRS.1.1 FDD |
|  | Config 2 |  | TRS.1.1 TDD |
|  | Config 3 |  | TRS.1.2 TDD |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD |
|  | Config 2 |  | CCR.1.1 TDD |
|  | Config 3 |  | CCR.2.1 TDD |
| OCNG Patterns | Config 1,2 |  | OP.1Note 7 |
|  | Config 3 |  | OP.1 Note 8 |
| SMTC Configuration |  |  | SMTC.1 |
| SSB Configuration | Config 1,2 |  | SSB.1 FR1 |
|  | Config 3 |  | SSB.2 FR1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| NocNote 2 |  | dBm/15 kHz | -104 |
| SS-RSRP Note 3 |  | dBm/15 kHz | -87 |
| Ês/Iot |  | dB | 17 |
| Ês/Noc |  | dB | 17 |
| NocNote 2 | Config 1,2 | dBm/SCS | -104 |
|  | Config 3 |  | -101 |
| IoNote3 | Config 1,2 | dBm/9.36MHz | -58.96 |
|  | Config 3 | dBm/38.16MHz | -52.86 |
| Time offset to Cell1 Note 5 |  | s | - |
| Propagation Condition |  |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modeled as AWGN of appropriate power for Noc to be fulfilled within BWoccupied. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselvess. Note 4: Void Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 6: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2 defined in clause 12 of TS 38.213 [3]. Note 7: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 8: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 9: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |

Table A.6.5.2.1.1-4: NR cell specific test parameters for NR SCell for interruptions during measurements on deactivated NR SCC in standalone NR

| Parameter |  | Unit | Cell2 |
| --- | --- | --- | --- |
| Frequency Range |  |  | FR1 |
| Duplex mode | ConfigSCell 1 |  | FDD |
|  | ConfigSCell 2,3 |  | TDD |
| TDD configuration | ConfigSCell 1 |  | Not Applicable |
|  | ConfigSCell 2 |  | TDDConf.1.1 |
|  | ConfiqSCell 3 |  | TDDConf.2.1 |
| BWchannel | ConfigSCell 1,2 |  | Note 9 |
|  | ConfigSCell 3 |  | Note 9 |
| BWoccupied | ConfigSCell 1,2 | RB | 52 Note 7 |
|  | ConfigSCell 3 |  | 106 Note 8 |
| Initial DL BWP Configuration | ConfigSCell 1,2,3 |  | DLBWP.0.1 |
| Dedicated DL BWP Configuration | ConfigSCell 1,2,3 |  | DLBWP.1.1 |
| Initial UL BWP Configuration | ConfigSCell 1,2,3 |  | N/A |
| Dedicated UL BWP Configuration | Config 1,2,3 |  | N/A |
| PDSCH Reference measurement channel | ConfigSCell 1 |  | SR.1.1 FDD |
|  | ConfigSCell 2 |  | SR.1.2 TDD |
|  | ConfigSCell 3 |  | SR.2.1 TDD |
| CSI-RS for tracking | ConfigSCell 1 |  | TRS.1.1 FDD |
|  | ConfigSCell 2 |  | TRS.1.1 TDD |
|  | ConfigSCell 3 |  | TRS.1.2 TDD |
| RMSI CORESET parameters | ConfigSCell 1 |  | CR.1.1 FDD |
|  | ConfigSCell 2 |  | CR.1.1 TDD |
|  | ConfigSCell 3 |  | CR.2.1 TDD |
| Dedicated CORESET parameters | ConfigSCell 1 |  | CCR.1.1 FDD |
|  | ConfigSCell 2 |  | CCR.1.1 TDD |
|  | ConfigSCell 3 |  | CCR.2.1 TDD |
| OCNG Patterns | ConfigSCell 1,2 |  | OP.1 Note 7 |
|  | ConfigSCell 3 |  | OP.1 Note 8 |
| SMTC Configuration |  |  | SMTC.4 |
| SSB Configuration | ConfigSCell 1,2 |  | SSB.5 FR1 |
|  | ConfigSCell 3 |  | SSB.6 FR1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| NocNote 2 |  | dBm/15 kHz | -104 |
| SS-RSRP Note 3 |  | dBm/15 kHz | -87 |
| Ês/Iot |  | dB | 17 |
| Ês/Noc |  | dB | 17 |
| NocNote 2 | ConfigSCell 1,2 | dBm/SCS | -104 |
|  | ConfigSCell 3 |  | -101 |
| IoNote3 | ConfigSCell 1,2 | dBm/9.36MHz | -58.96 |
|  | ConfigSCell 3 | dBm/38.16MHz | -52.86 |
| Time offset to Cell1 Note 5 |  | s | 3 |
| Propagation Condition |  |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modeled as AWGN of appropriate power for Noc to be fulfilled within BWoccupied. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselvess. Note 4: Void Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 6: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2 defined in clause 12 of TS 38.213 [3]. Note 7: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 8: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 9: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |

##### A.6.5.2.1.2	Test Requirements

The UE shall be continuously scheduled on PCell during the entire length of T1. During the time duration T1 the UE shall transmit at least 99.5% of ACK/NACK on PCell.

If the NR PCell is not in the same band as the deactivated SCell, the UE is only allowed to cause interruptions on NR PCell immediately before and immediately after an SMTC. Each interruption on NR PCell shall not exceed the value defined in Table A.6.5.2.1.2-1.

If the NR PCell is in the same band as the deactivated SCell, the UE is only allowed to cause an interruption on PCell no earlier than 1 slot before an SMTC and no later than 1 slot after the SMTC. the interruption on NR PCell shall not exceed the value defined in Table A.6.5.2.1.2-2.

Table A.6.5.2.1.2-1: Interruption duration if the PCell is not in the same band as the deactivated SCell

|  | NR Slot length (ms) | Interruption length |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |

Table A.6.5.2.1.2-2: Interruption duration if the PCell is in the same band as the deactivated SCell

|  | NR Slot length (ms) | Interruption length |
| --- | --- | --- |
| 0 | 1 | 2 + SMTC duration |
| 1 | 0.5 | 2 + SMTC duration |

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.2.2	SA interruptions at NR SRS carrier based switching

##### A.6.5.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that when a UE needs to transmit aperiodic SRS, the UE can perform carrier based switching to one carrier not configured for PUCCH/PUSCH transmission from a carrier with PUCCH/PUSCH transmission. The test will partly verify the interruption requirements on PCell in clause 8.2.2.2.9.

##### A.6.5.2.2.2	Test Parameters

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the FR1 PCell and Cell 2 is activated SCell on the TDD SCC which operats in downlink without PUCCH/PUSCH. The UE is configured with the SRS switching between PCell and SCell.The test parameters for PCell and SCell are given in Table A.6.5.2.2.2-2 and A.6.5.2.2.2-3 below. The test consists of two successive time periods, with duration of T1 and T2, respectively. Immediately at the beginning of T2, the UE is triggered for SRS switching.

The test equipment verifies that potential interruption is carried out correctly by monitoring ACK/NACK sent in PCell.

Table A.6.5.2.2.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD – TDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD – TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD – TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.5.2.2.2-2: General test parameters for SA interruptions at NR SRS carrier based switching

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channel (1, 2) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1 |
| Configured SCell |  | Cell 2 | Activated secondary cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 5 |  |
| T2 | ms | 40 | UE shall perform SRS switching during T2 |

Table A.6.5.2.2.2-3: Cell specific test parameters for SA interruptions at NR SRS carrier based switching

| Parameter |  |  |  | Unit | T1 |  | T2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Duplex mode |  | Config 1 |  |  | FDD | TDD | FDD | TDD |
|  |  | Config 2,3 |  |  | TDD |  |  |  |
| TDD configuration |  | Config 1 |  |  | N/A | TDDConf.1.1 | N/A | TDDConf.1.1 |
|  |  | Config 2 |  |  | TDDConf.1.1 |  |  |  |
|  |  | Config 3 |  |  | TDDConf.2.1 |  |  |  |
| BWchannel |  | Config 1,2 |  | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3 |  |  | 40: NRB,c = 106 |  |  |  |
| Downlink initial BWP Configuration |  |  |  |  | DLBWP.0.1 |  |  |  |
| Downlink dedicated BWP Configuration |  |  |  |  | DLBWP.1.1 |  |  |  |
| Uplink initial BWP configuration |  |  |  |  | ULBWP.0.1 |  |  |  |
| Uplink dedicated BWP configuration |  |  |  |  | ULBWP.1.1 |  |  |  |
| TCI state |  |  |  |  | TCI.State.0 |  |  |  |
| TRS Configuration |  |  |  |  | TRS.1.1 TDD |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  |  | SR.1.1 FDD | SR.1.1 TDD | SR.1.1 FDD | SR.1.1 TDD |
|  |  | Config 2 |  |  | SR.1.1 TDD | SR.1.1 TDD | SR.1.1 TDD | SR.1.1 TDD |
|  |  | Config 3 |  |  | SR2.1 TDD | SR2.1 TDD | SR2.1 TDD | SR2.1 TDD |
| Dedicated CORESET parameters |  | Config 1 |  |  | CCR.1.1 FDD | CCR.1.1 TDD | CCR.1.1 FDD | CCR.1.1 TDD |
|  |  | Config 2 |  |  | CCR.1.1 TDD | CCR.1.1 TDD | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | Config 3 |  |  | CCR.2.1 TDD | CCR.2.1 TDD | CCR.2.1 TDD | CCR.2.1 TDD |
| RMSI CORESET parameters |  | Config 1 |  |  | CR.1.1 FDD | CR.1.1 TDD | CR.1.1 FDD | CR.1.1 TDD |
|  |  | Config 2 |  |  | CR.1.1 TDD | CR.1.1 TDD | CR.1.1 TDD | CR.1.1 TDD |
|  |  | Config 3 |  |  | CR2.1 TDD | CR2.1 TDD | CR2.1 TDD | CR2.1 TDD |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |  |
| SRS Configuration | Config 1,2 |  |  |  | SRS.1 TDD |  |  |  |
|  | Config 3 |  |  |  | SRS.2 TDD |  |  |  |
| SSB Configuration | Config 1,2 |  |  |  | SSB.1 FR1 |  |  |  |
|  | Config 3 |  |  |  | SSB.2 FR1 |  |  |  |
| SMTC configuration |  |  |  |  | SMTC.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  |  | Config 1,2,4,5 | dBm/15kHz | -104 |  |  |  |
|  |  |  | Config 3,6 |  | -101 |  |  |  |
|  |  |  |  | dB | 17 |  |  |  |
|  |  |  |  | dB | 17 |  |  |  |
| SS-RSRPNote3 |  |  | Config 1,2,4,5 | dBm/SCS | -87 |  |  |  |
|  |  |  | Config 3,6 |  | -84 |  |  |  |
| SCH_RP Note 3 |  |  |  | dBm/15 kHz | -87 |  |  |  |
| Propagation condition |  |  |  | - | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |  |  |

Table A.6.5.2.2.2-4: Void

##### A.6.5.2.2.3	Test Requirements

The UE shall be scheduled on PCell continuously throughout the test. During the time duration T2, the interruption on PCell shall not be more than the values specified for SA in clause 8.2.2.2.9.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.2.3	SA interruptions at NR SRS antenna port switching with 1 SRS symbol in a slot in NR-CA

##### A.6.5.2.3.1	Test Purpose and Environment

The purpose of this test is to verify that when a UE performs SRS antenna port switching, i.e. transmits SRS on the antenna port(s) not used for PUCCH/PUSCH transmission and on the antenna port(s) used for PUCCH/PUSCH transmission at different SRS transmission occasions. The test will partly verify the interruption requirements on PCell and SCell in clause 8.2.2.2.16. The interruption requirement is defined based on the band combination capability reported by UE, i.e., based on txSwitchImpactToRx or txSwitchWithAnotherBand as specified in requirement applicability in clause 8.2.2.2.16.

##### A.6.5.2.3.2	Test Parameters

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the FR1 PCell and Cell 2 is activated SCell on the TDD PCC. Only PCC may be configured with more than 1 SRS resources in each SRS resource set with usage set to ‘antennaSwitching’. The test parameters for PCell and SCell are given in Table A.6.5.2.3.2-2 and A.6.5.2.3.2-3 below. The test consists of two successive time periods, with duration of T1 and T2, respectively. Immediately at the beginning of T2, the UE is configured with periodic SRS for antenna port switching via RRC reconfiguration. Note that the RRC reconfiguration message should be sent to UE at the time 50ms before the beginning of T2.

The test equipment verifies that potential interruption is carried out correctly by monitoring ACK/NACK sent in SCell.

Table A.6.5.2.3.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note1: The UE is only required to be tested in one of the supported test configurations. Note2: Test configuration for NR PCell and NR SCell are chosen independently. |  |

Table A.6.5.2.3.2-2: General test parameters for SA interruptions at NR SRS antenna switching

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channel (1, 2) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1 |
| Configured SCell |  | Cell 2 | Activated secondary cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 5 |  |
| T2 | ms | 40 | UE shall perform SRS antenna switching during T2 |

Table A.6.5.2.3.2-3: Cell specific test parameters for SA interruptions at NR SRS antenna switching

| Parameter |  |  |  | Unit | T1 |  | T2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Duplex mode |  | Config 1 |  |  | FDD |  |  |  |
|  |  | Config 2,3 |  |  | TDD |  |  |  |
| TDD configuration |  | Config 1 |  |  | N/A |  |  |  |
|  |  | Config 2 |  |  | TDDConf.1.2 |  |  |  |
|  |  | Config 3 |  |  | TDDConf.2.3 |  |  |  |
| BWchannel |  | Config 1,2 |  | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3 |  |  | 40: NRB,c = 106 |  |  |  |
| Downlink initial BWP Configuration |  |  |  |  | DLBWP.0.1 |  |  |  |
| Downlink dedicated BWP Configuration |  |  |  |  | DLBWP.1.1 |  |  |  |
| Uplink initial BWP configuration |  |  |  |  | ULBWP.0.1 |  |  |  |
| Uplink dedicated BWP configuration |  |  |  |  | ULBWP.1.1 |  |  |  |
| TCI state |  |  |  |  | TCI.State.0 |  |  |  |
| TRS Configuration |  |  |  |  | TRS.1.1 TDD |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  |  | SR.1.1 FDD | SR.1.1 FDD | SR.1.1 FDD | SR.1.1 FDD |
|  |  | Config 2 |  |  | SR.1.1 TDD | SR.1.1 TDD | SR.1.1 TDD | SR.1.1 TDD |
|  |  | Config 3 |  |  | SR2.1 TDD | SR2.1 TDD | SR2.1 TDD | SR2.1 TDD |
| Dedicated CORESET parameters |  | Config 1 |  |  | CCR.1.1 FDD | CCR.1.1 FDD | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | Config 2 |  |  | CCR.1.1 TDD | CCR.1.1 TDD | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | Config 3 |  |  | CCR.2.1 TDD | CCR.2.1 TDD | CCR.2.1 TDD | CCR.2.1 TDD |
| RMSI CORESET parameters |  | Config 1 |  |  | CR.1.1 FDD | CR.1.1 FDD | CR.1.1 FDD | CR.1.1 FDD |
|  |  | Config 2 |  |  | CR.1.1 TDD | CR.1.1 TDD | CR.1.1 TDD | CR.1.1 TDD |
|  |  | Config 3 |  |  | CR2.1 TDD | CR2.1 TDD | CR2.1 TDD | CR2.1 TDD |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |  |
| SRS Configuration | Config 1,2 |  |  |  | SRS.X1, with dedicated SRS configuation in Table A.6.5.2.3.2-4 |  |  |  |
|  | Config 3 |  |  |  | SRS.X2, with dedicated SRS configuation in Table A.6.5.2.3.2-4 |  |  |  |
| SSB Configuration | Config 1,2 |  |  |  | SSB.1 FR1 |  |  |  |
|  | Config 3 |  |  |  | SSB.2 FR1 |  |  |  |
| SMTC configuration |  |  |  |  | SMTC.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 2) |  |  |  |  |  |  |  |  |
| Note3 |  |  | Config 1,2,4,5 | dBm/15kHz | -104 |  |  |  |
|  |  |  | Config 3,6 |  | -101 |  |  |  |
|  |  |  |  | dB | 17 |  |  |  |
|  |  |  |  | dB | 17 |  |  |  |
| SS-RSRPNote4 |  |  | Config 1,2,4,5 | dBm/SCS | -87 |  |  |  |
|  |  |  | Config 3,6 |  | -84 |  |  |  |
| SCH_RP Note 4 |  |  |  | dBm/15 kHz | -87 |  |  |  |
| Propagation condition |  |  |  | - | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Test configuration for NR PCell and NR SCell are chosen independently. |  |  |  |  |  |  |  |  |

Table A.6.5.2.3.2-4: Specific Sounding Reference Symbol Configuration for xTyR configuration

| Parameter |  | xTyR configuration |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1T2R |  | 2T4R |  | 1T4R |  |  |  |
| srs-ResourceId |  | 0 | 1 | 0 | 1 | 0 | 1 | 2 | 3 |
| startPosition |  | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| nrofSRS-Ports |  | port1 | port1 | port2 | port2 | port1 | port1 | port1 | port1 |
| nrofSymbols |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| periodicityAndOffset-p | Config 1,2 | sl40, 1 | sl40, 6 | sl40, 1 | sl40, 6 | sl40, 1 | sl40, 6 | sl40, 11 | sl40, 16 |
|  | Config 3 | sl80, 3 | sl80, 13 | sl80, 3 | sl80, 13 | sl80, 3 | sl80, 13 | sl80, 23 | sl80, 33 |

##### A.6.5.2.3.3	Test Requirements

The UE shall be scheduled on SCell continuously throughout the test.

During the time duration T2, the DL interruption on NR SCell during the SRS antenna switching in each SRS transmission slot on NR PCell shall not exceed 1 slot if SCell is indicated in txSwitchImpactToRx.

During the time duration T2, the UL interruption on NR SCell during the SRS antenna switching in each SRS transmission slot on NR PCell shall not exceed 1 slot if SCell is indicated in txSwitchWithAnotherBand.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.2.4	SA interruptions at NR SRS antenna port switching

##### A.6.5.2.4.1	Test Purpose and Environment

The purpose of this test is to verify the interruption requirement on victim CC during SRS antenna port switching with more than 1 SRS symbols on aggressor CC defined in clause 8.2.2.2.16. The interruption requirement is defined based on the band combination capability reported by UE, i.e., based on txSwitchImpactToRx or txSwitchWithAnotherBand as specified in requirement applicability in clause 8.2.2.2.16.

##### A.6.5.2.4.2	Test Parameters

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the FR1 PCell and Cell 2 is FR1 SCell. The UE is configured with the SRS antenna port in FR1 PCell. The test parameters for PCell and SCell are given in Table A.6.5.2.4.2-2 and A.6.5.2.4.2-3 below. Common SRS configuration is given in A.3.24. Dedicated SRS configuration which is dependent on reported SRS capability supportedSRS-TxPortSwitch, is given in Table A.6.5.2.4.2-4. The test consists of two successive time periods, with duration of T1 and T2, respectively. Immediately at the beginning of T2, the UE is triggered for SRS antenna port switching.

The test equipment verifies that potential interruption is carried out correctly by monitoring ACK/NACK sent in PCell.

Table A.6.5.2.4.2-1: Supported test configurations

| Configuration | Description |  |
| --- | --- | --- |
|  | NR PCell Note 2 | NR SCell Note 2 |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD – TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, FDD – TDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD – TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, TDD – TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD – TDD duplex mode | 30 kHz SSB SCS, 40 MHz bandwidth, TDD – TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: Test configuration for NR PCell and NR SCell are chosen independently. |  |  |

Table A.6.5.2.4.2-2: General test parameters for SA interruptions at NR SRS antenna port switching

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channel (1, 2) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1 |
| Configured SCell |  | Cell 2 | Activated secondary cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 5 |  |
| T2 | ms | 40 | UE shall perform SRS antenna port switching during T2 |

Table A.6.5.2.4.2-3: Cell specific test parameters for SA interruptions at NR SRS antenna port switching

| Parameter |  |  |  | Unit | T1 |  |  | T2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 |  | Cell 2 |  |
| Duplex mode |  | Config 1 |  |  | FDD | TDD |  | FDD |  | TDD |  |
|  |  | Config 2,3 |  |  | TDD |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  |  | N/A | TDDConf.1.2 |  | N/A |  | TDDConf.1.2 |  |
|  |  | Config 2 |  |  | TDDConf.1.2 |  |  |  |  |  |  |
|  |  | Config 3 |  |  | TDDConf.2.3 |  |  |  |  |  |  |
| BWchannel |  | Config 1,2 |  | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| Downlink initial BWP Configuration |  |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |
| Downlink dedicated BWP Configuration |  |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |
| TCI state |  |  |  |  | TCI.State.0 |  |  |  |  |  |  |
| TRS Configuration |  |  |  |  | TRS.1.1 TDD |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  |  | SR.1.1 FDD | SR.1.1 TDD |  | SR.1.1 FDD |  | SR.1.1 TDD |  |
|  |  | Config 2 |  |  | SR.1.1 TDD | SR.1.1 TDD |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  | Config 3 |  |  | SR2.1 TDD | SR2.1 TDD |  | SR2.1 TDD |  | SR2.1 TDD |  |
| Dedicated CORESET parameters |  | Config 1 |  |  | CCR.1.1 FDD | CCR.1.1 TDD |  | CCR.1.1 FDD |  | CCR.1.1 TDD |  |
|  |  | Config 2 |  |  | CCR.1.1 TDD | CCR.1.1 TDD |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  |  | CCR.2.1 TDD | CCR.2.1 TDD |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| RMSI CORESET parameters |  | Config 1 |  |  | CR.1.1 FDD | CR.1.1 TDD |  | CR.1.1 FDD |  | CR.1.1 TDD |  |
|  |  | Config 2 |  |  | CR.1.1 TDD | CR.1.1 TDD |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | Config 3 |  |  | CR2.1 TDD | CR2.1 TDD |  | CR2.1 TDD |  | CR2.1 TDD |  |
| ptrs-portindex |  |  |  |  | NA. | n0 |  | NA. |  | n1 |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |  |  |  |  |
| SSB Configuration | Config 1,2 |  |  |  | SSB.1 FR1 |  |  |  |  |  |  |
|  | Config 3 |  |  |  | SSB.2 FR1 |  |  |  |  |  |  |
| SRS configuration | Config 1,2 |  |  |  | SRS.x1 Note 4 |  | - |  | SRS.x1 Note 4 |  | - |
|  | Config 3 |  |  |  | SRS.x2 Note 4 |  | - |  | SRS.x2 Note 4 |  | - |
| SMTC configuration |  |  |  |  | SMTC.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | Config 1,2 | dBm/15kHz | -104 |  |  |  |  |  |  |
|  |  |  | Config 3 |  | -101 |  |  |  |  |  |  |
|  |  |  |  | dB | 17 |  |  |  |  |  |  |
|  |  |  |  | dB | 17 |  |  |  |  |  |  |
| SS-RSRPNote3 |  |  | Config 1,2 | dBm/SCS | -87 |  |  |  |  |  |  |
|  |  |  | Config 3 |  | -84 |  |  |  |  |  |  |
| SCH_RP Note 3 |  |  |  | dBm/15 kHz | -87 |  |  |  |  |  |  |
| Propagation condition |  |  |  | - | AWGN |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Dedicated SRS configuration in Table A.6.5.2.4.2-4 |  |  |  |  |  |  |  |  |  |  |  |

Table A.6.5.2.4.2-4: Dedicated Sounding Reference Symbol Configuration for xTyR configuration

| Parameter |  | xTyR configuration |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1T2R |  | 2T4R |  | 1T4R |  |  |  |
| srs-ResourceId |  | 0 | 1 | 0 | 1 | 0 | 1 | 2 | 3 |
| nrofSymbols |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| startPosition |  | 5 | 3 | 5 | 3 | 5 | 3 | 5 | 3 |
| nrofSRS-Ports |  | port1 | port1 | port2 | port2 | port1 | port1 | port1 | port1 |
| periodicityAndOffset-p | Config 1,2 | sl40, 1 | sl40, 1 | sl40, 1 | sl40, 1 | sl40, 1 | sl40, 1 | sl40, 6 | sl40, 6 |
|  | Config 3 | sl80, 3 | sl80, 3 | sl80, 3 | sl80, 3 | sl80, 3 | sl80, 3 | sl80, 13 | sl80, 13 |

##### A.6.5.2.4.3	Test Requirements

The UE shall be scheduled on PCell continuously throughout the test. During the time duration T2, the interruption on SCell shall not be more than the values specified in Table 8.2.1.2.18-3 in clause 8.2.1.2.18 for each SRS transmission slot.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.6.5.3	SCell Activation and Deactivation Delay

#### A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle

##### A.6.5.3.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in clause 8.3, when the SCell in FR1 is known by the UE at the time of activation.

The supported test configurations for NR PCell are shown in table A.6.5.3.1.1-1 below. Supported test configurations for NR SCell are shown in table A.6.5.3.1.1-1A. Test configuration for NR PCell and test configuration for NR SCell are chosen independently. The test parameters are given in Tables A.6.5.3.1.1-2 and cell-specific parameters in A.6.5.3.1.1-3 and A.6.5.3.1.1-4 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two NR carriers, each with one cell. Both cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1, but is not aware of Cell2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2. The UE now starts monitoring the SCC. The test equipment sends a MAC message for activation of the SCell.

The point in time at which the MAC message is received at the UE antenna connector, in slot # denoted n, defines the start of time period T2. The UE shall be able to report valid CSI in PCell for the activated SCell at latest in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length, as defined in clause 8.3. The UE shall start reporting CSI in PCell after at least one CSI-RS transmission occasion for channel measurement and reporting after slot n+THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3, where Ninterruption is the interruption length given in clause 8.2.

Time period T3 starts when a MAC message for deactivation of SCell, sent from the test equipment to the UE in a slot # denoted m, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell in a slot m+THARQ+3msNR slot length, as defined in clause 8.3, and The starting point of any PCell interruption due to the deactivation shall occur in the slot m+1+THARQNR slot length to m+1+THARQ+3msNR slot length, as defined in clause 8.3.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the SCell deactivation command is sent until CQI reporting for SCell is discontinued.

Table A.6.5.3.1.1-1: known FR1 SCell activation in non-DRX for 160ms SCell measurement cycle supported test configurations for NR PCell

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table A.6.5.3.1.1-1A: known FR1 SCell activation in non-DRX for 160ms SCell measurement cycle supported test configurations for NR SCell

| ConfigSCell | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table A.6.5.3.1.1-2: General test parameters for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channel (1, 2) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 7 | During this time the PSCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| THARQ | ms | Config 1: 2 Config 2: 3 Config 3: 2.5 | k1×NR slot length k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [3] that will meet the timing constraints of this test case. |
| TCSI_Reporting | ms | 15 | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |

Table A.6.5.3.1.1-3: Cell specific test parameters for NR PCell for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter |  | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| Duplex mode | Config 1 |  | FDD |  |  |
|  | Config 2,3 |  | TDD |  |  |
| TDD configuration | Config 1 |  | Not applicable |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |
| BWchannel | Config 1,2 | MHz | Note 7 |  |  |
|  | Config 3 |  | Note 7 |  |  |
| BWoccupied | Config 1,2 | RB | 52 Note 5 |  |  |
|  | Config 3 |  | 106 Note 6 |  |  |
| Initial BWP configuration |  |  | DLBWP.0.1 |  |  |
| TCI state |  |  | TCI.State.0 |  |  |
| TRS Configuration | Config 1 |  | TRS.1.1 FDD |  |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD |  |  |
|  | Config 2 |  | CCR.1.1 TDD |  |  |
|  | Config 3 |  | CCR.2.1 TDD |  |  |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  |
| OCNG Patterns | Config 1,2 |  | OP.1Note 5 |  |  |
|  | Config 3, |  | OP.1 Note 6 |  |  |
| SSB Configuration | Config 1,2 |  | SSB.1 FR1 |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |
| CSI-RS configuration for CSI reporting (Note 8) | Config 1 |  | CSI-RS.1.1 FDD |  |  |
|  | Config 2 |  | CSI-RS.1.1 TDD |  |  |
|  | Config 3 |  | CSI-RS.2.1 TDD |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |
| reportConfigType |  |  | periodic |  |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |  |
| CSI reporting periodicity | Config 1,2 | slot | 5 |  |  |
|  | Config 3 |  | 10 |  |  |
| CSI reporting offset | Config 1,2 | slot | 3 |  |  |
|  | Config 3 |  | 5 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | Config 1,2 | dBm/SCS | -104 |  |  |
|  | Config 3 |  | -101 |  |  |
|  |  | dB | 17 |  |  |
|  |  | dB | 17 |  |  |
| SS-RSRPNote3 | Config 1,2 | dBm/SCS | -87 |  |  |
|  | Config 3 |  | -84 |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -87 |  |  |
| Io Note3 | Config 1,2 | dBm/ 9.36MHz | -58.96 |  |  |
|  | Config 3 | dBm/ 38.16MHz | -52.87 |  |  |
| Propagation condition |  | - | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. Note 3: SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. Note 8: On top of the reference configurations, CSI-RS offset should be set to meet the CSI reference resource timing definition in TS 38.214 cl. 5.2.2.5. |  |  |  |  |  |

Table A.6.5.3.1.1-4: Cell specific test parameters for NR SCell for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter |  | Unit | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| Duplex mode | ConfigSCell 1 |  | FDD |  |  |
|  | ConfigSCell 2,3 |  | TDD |  |  |
| TDD configuration | ConfigSCell 1 |  | Not applicable |  |  |
|  | ConfigSCell 2 |  | TDDConf.1.1 |  |  |
|  | ConfigSCell 3 |  | TDDConf.2.1 |  |  |
| BWchannel | ConfigSCell 1,2 | MHz | Note 7 |  |  |
|  | ConfigSCell 3 |  | Note 7 |  |  |
| BWoccupied | ConfigSCell 1,2 | RB | 52 Note 5 |  |  |
|  | ConfigSCell 3 |  | 106 Note 6 |  |  |
| Initial BWP configuration |  |  | DLBWP.0.1 |  |  |
| TCI state |  |  | TCI.State.0 |  |  |
| TRS Configuration | ConfigSCell 1 |  | TRS.1.1 FDD |  |  |
|  | ConfigSCell 2 |  | TRS.1.1 TDD |  |  |
|  | ConfigSCell 3 |  | TRS.1.2 TDD |  |  |
| PDSCH Reference measurement channel | ConfigSCell 1 |  | N/A |  |  |
|  | ConfigSCell 2 |  | N/A |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |
| Dedicated CORESET parameters | ConfigSCell 1 |  | N/A |  |  |
|  | ConfigSCell 2 |  | N/A |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |
| RMSI CORESET parameters | ConfigSCell 1 |  | N/A |  |  |
|  | ConfigSCell 2 |  | N/A |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |
| OCNG Patterns | ConfigSCell 1,2 |  | OP.1Note 5 |  |  |
|  | ConfigSCell 3, |  | OP.1 Note 6 |  |  |
| SSB Configuration | ConfigSCell 1,2 |  | SSB.1 FR1 |  |  |
|  | ConfigSCell 3 |  | SSB.2 FR1 |  |  |
| CSI-RS configuration for CSI reporting Note 8 | ConfigSCell 1 |  | CSI-RS.1.1 FDD |  |  |
|  | ConfigSCell 2 |  | CSI-RS.1.1 TDD |  |  |
|  | ConfigSCell 3 |  | CSI-RS.2.1 TDD |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |
| reportConfigType |  |  | N/A |  |  |
| reportQuantity |  |  | N/A |  |  |
| CSI reporting periodicity | ConfigSCell 1,2 | slot | N/A |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |
| CSI reporting offset | ConfigSCell 1,2 | slot | N/A |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | ConfigSCell 1,2 | dBm/SCS | -104 |  |  |
|  | ConfigSCell 3 |  | -101 |  |  |
|  |  | dB | 17 |  |  |
|  |  | dB | 17 |  |  |
| SS-RSRPNote3 | ConfigSCell 1,2 | dBm/SCS | -87 |  |  |
|  | ConfigSCell 3 |  | -84 |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -87 |  |  |
| Io Note3 | ConfigSCell 1,2 | dBm/ 9.36MHz | -58.96 |  |  |
|  | ConfigSCell 3 | dBm/ 38.16MHz | -52.87 |  |  |
| Propagation condition |  | - | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. Note 3: SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. Note 8: On top of the reference configurations, CSI-RS offset should be set to meet the CSI reference resource timing definition in TS 38.214 cl. 5.2.2.5. |  |  |  |  |  |

##### A.6.5.3.1.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell in the first available uplink resource after at least one CSI-RS transmission occasion for channel measurement and reporting after slot (n+1+THARQ+3msNR slot length). UE is allowed to postpone CSI report to next available UL resource if an available uplink resource is subject to interruption.

During T3 the UE shall stop sending CSI reports for SCell at latest in a slot m+THARQ+3msNR slot length, as defined in clause 8.3.

During T2 interruption of PCell / PSCell during SCell activation shall not happen outside the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3.

During T3 the starting point of interruption of PCell during SCell deactivation shall not happen outside the slot m+1+THARQNR slot length to m+1+THARQ+3msNR slot length, as defined in clause 8.3.

The interruption on any activated serving cell shall not be more than the values specified for SA in clause 8.2.2.2.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay and SCell deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a slot THARQ+Tactivtion_time+TCSI_ReportingNR slot length as defined in clause 8.3 then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

#### A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle

##### A.6.5.3.2.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in clause A.6.5.3.1.1. The supported test configurations are the same as defined in clause A.6.5.3.1.1. The test parameters are the same except those described in the following clause. The listed parameter values in Tables A.6.5.3.2.1-1 will replace the values of corresponding parameters in Tables A.4.5.3.1.1-1.

Table A.6.5.3.2.1-1: General test parameters for known FR1 SCell activation case, 640 ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| SCell measurement cycle (measCycleSCell) | ms | 640 |  |

##### A.6.5.3.2.2	Test Requirements

The test requirements defined in clause A.6.5.3.1.2 shall apply to this test case, except Tactivation_time will be replaced with the value TFirstSSB_MAX + Trs + 5ms.

#### A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX

##### A.6.5.3.3.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in clause 8.3, when the SCell in FR1 is known by the UE at the time of activation.

The supported test configurations are shown in table A.6.5.3.1.1-1 below. The test parameters are given in Tables A.6.5.3.1.1-2 and cell-specific parameters in A.6.5.3.1.1-3 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two NR carriers, each with one cell. Both cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1, but is not aware of Cell2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2. The UE now starts monitoring the SCC. The test equipment sends a MAC message for activation of the SCell.

The point in time at which the MAC message is received at the UE antenna connector, in slot # denoted n, defines the start of time period T2. The UE shall be able to report valid CSI in PCell for the activated SCell at latest in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length, as defined in clause 8.3. The UE shall start reporting CSI in PCell after at least one CSI-RS transmission occasion for channel measurement and reporting after slot n+THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3, where Ninterruption is the interruption length given in clause 8.2.

Time period T3 starts when a MAC message for deactivation of SCell, sent from the test equipment to the UE in a slot # denoted m, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell in a slot m+THARQ+3msNR slot length, as defined in clause 8.3, and The starting point of any PCell interruption due to the deactivation shall occur in the slot m+1+THARQNR slot length to m+1+THARQ+3msNR slot length, as defined in clause 8.3.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the SCell deactivation command is sent until CQI reporting for SCell is discontinued.

Table A.6.5.3.3.1-1: General test parameters for unknown FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| T1 | ms | 100 | During this time the PSCell shall be known and the SCell configured, but not detected. |

##### A.6.5.3.3.2	Test Requirements

The test requirements defined in clause A.6.5.3.1.2 shall apply to this test case, except Tactivation_time will be replaced with the value TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms as defined in clause 8.3.

#### A.6.5.3.4	Direct SCell activation at SCell addition of known SCell in FR1

##### A.6.5.3.4.1	Test Purpose and Environment

The purpose of this test is to verify fulfillment of direct SCell activation delay and interruption requirements at SCell addition as defined in clause 8.3.4 and 8.2.2, respectively. The supported test configurations are shown in Table A.6.5.3.4.1-1.

The test scenario comprises one PCell (Cell 1) and one SCell (Cell 2) as outlined in Table A.6.5.3.4.1-2. Cell-specific parameters are provided in Table A.6.5.3.4.1-3.

The test consists of two successive time periods with duration T1 and T2, respectively. There are two carriers, each with one cell. Cell 1 (PCell) is on RF channel 1 (PCC), and Cell 2 (SCell) is on RF channel 2 (SCC). Cell 1 and Cell 2 both operate according to one of the configurations in Table A.6.5.3.4.1-1.

Before the test starts the UE is connected to Cell 1 on RF channel 1. The UE is only monitoring RF channel 1 and is not aware of Cell 2 on RF channel 2.

The UE is continuously scheduled in PCell throughout the test.

At the beginning of T1 the UE is configured to measure RF channel 2 in measurement gaps. During T1, the UE detects and measures Cell 2 on RF channel 2, and sends a measurement report containing Cell 2 to the test equipment. After having received a measurement report containing Cell 2, the test equipment deconfigures the measurement gaps and thereafter sends a RRC connection reconfiguration message to the UE by which it configures the SCell (Cell 2) in activated state (sCellState is set to activated). The time between reception of the last measurement report carrying SCell and transmission of the RRC connection reconfiguration message directly activating SCell is kept short enough to allow the SCell to remain known to the UE.

Time period T2 starts when the UE receives the RRC connection reconfiguration message at the UE antenna connector. The corresponding slot at which the message is received at the UE antenna connector is denoted n. The UE shall complete activation of the SCell no later than in slot n + NdirectNR slot length, as specified in clause 8.3.4. From slot n+ NdirectNR slot length and onwards the UE shall report valid CSI both for PCell and SCell.

The test equipment verifies the activation time by counting the slots between the RRC connection reconfiguration message is sent and until CSI report with non-zero CQI for both PCell and SCell is received.

The test equipment verifies that interruptions on other serving cells are within the requirements by counting ACK/NACKs transmitted in PCell.

Table A.6.5.3.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Note: The UE is only required to be tested in one of the supported test configurations

Table A.6.5.3.4.1-2: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1, 2 | Two NR radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Inter-frequency neighbor cell (SCell to-be) |  | Cell 2 | Inter-frequency neighbor cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Measurement gap pattern |  | gp0 | Measurement gap is used during parts of time period T1 for detection of Cell 2. |
| CSI reporting periodicity | ms | 2 | CSI reporting periodicity for periodic reporting of CQI for PCell and, when added, SCell. |
| SCell measurement cycle (measCycleSCell) | ms | 160 | Measurement cycle for SCell does not come into effect in direct activation at SCell addition. |
| Timing offset between Cell 1 and Cell 2 | s | MRTD | The value of maximum timing offset depends upon the carrier aggregation scenario. |
| T1 | s | 7 | During this time period the PCell shall be known and Cell 2 shall be detected as an inter-frequency neighbor cell. |
| T2 | s | 1 | During this time period Cell 2 shall be configured and directly activated as SCell. |

Table A.6.5.3.4.1-3: NR Cell specific test parameters

| Parameter |  |  | Unit | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 |  | T2 |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |
| TDD configuration |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | Config 1,2 | MHz | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| BWP configuration |  | Initial DL |  | DLBWP.0.1 |  |  | --- | DLBWP.0.1 |
|  |  | Initial UL |  | ULBWP.0.1 |  |  |  | --- |
|  |  | Dedicated DL |  | DLBWP.1.1 |  |  |  | DLBWP.1.1 |
|  |  | Dedicated UL |  | ULBWP.1.1 |  |  |  | --- |
| TCI state |  |  |  | TCI.State.0 |  |  | --- | TCI.State.0 |
| CSI-RS configuration for CSI reporting |  | Config 1 |  | CSI-RS.1.1 FDD |  |  | --- | CSI-RS.1.1 FDD |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |  |  |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |  |  |  | CSI-RS.2.1 TDD |
| TRS Configuration |  | Config 1 |  | TRS.1.1 FDD |  |  | --- | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  | --- | SR.1.1 FDD |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |
|  |  | Config 3 |  | SR.2.1 TDD |  |  |  | SR.2.1 TDD |
| Dedicated CORESET parameters |  | Config 1 |  | CCR.1.1 FDD |  |  | --- | CCR.1.1 FDD |
|  |  | Config 2 |  | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |
|  |  | Config 3 |  | CCR.2.1 TDD |  |  |  | CCR.2.1 TDD |
| RMSI CORESET parameters |  | Config 1 |  | CR.1.1 FDD |  |  | --- |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |  |  |  |
| OCNG Pattern |  |  |  | OP.1 |  |  | OP.1 |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  | SSB.2 FR1 |  |
| SMTC configuration |  |  |  | SMTC.1 |  |  | SMTC.1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note1 |  |  |  |  |  |  |  |  |
| Noc Note2 | Config 1,2 |  | dBm/15kHz | -104 |  |  | -104 |  |
|  | Config 3 |  |  | -101 |  |  | -101 |  |
| Ês/Iot |  |  | dB | 17 |  |  | 17 |  |
| Ês/Noc |  |  | dB | 17 |  |  | 17 |  |
| SS-RSRP Note3 | Config 1,2 |  | dBm/SCS | -87 |  |  | -87 |  |
|  | Config 3 |  |  | -84 |  |  | -84 |  |
| Io Note3 | Config 1,2 |  | dBm/9.36 MHz | -59.0 |  |  | -59.0 |  |
|  | Config 3 |  | dBm/38.16 MHz | -52.9 |  |  | -52.9 |  |
| Propagation condition |  |  |  | AWGN |  |  | AWGN |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 1x2 Low |  |  | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP, SCH_RP, and Io levels have been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

##### A.6.5.3.4.2	Test Requirements

The UE shall complete the direct activation of the SCell no later than at slot n + NdirectNR slot length.

The UE shall report non-zero CQI for SCell from slot n + NdirectNR slot length and onwards throughout time period T2.

The interruption on PCell during direct activation of the SCell shall occur within the interruption window specified in clause 8.3.4 and shall not exceed the length specified in clause 8.2.2.2.11.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.3.5	Direct SCell activation at handover with known SCell in FR1

##### A.6.5.3.5.1	Test Purpose and Environment

This test is to verify the requirement for the FDD-FDD and TDD-TDD intra frequency handover with direct SCell activation requirements specified in subclause 8.3.5.

Supported test configurations for NR PCell are shown in table A.6.5.3.5.1-1. Supported test configurations for NR SCell are shown in table A.6.5.3.5.1-1A. Test configuration for NR PCell and test configuration for NR SCell are chosen independently. Both handover with direct SCell activation requirements are tested by using the parameters in table A.6.5.3.5.1-2, A.6.5.3.5.1-3 and A.6.5.3.5.1-4.

The test scenario comprises of three NR FDD or NR TDD FR1 carriers and the 3 cells as given in tables A.6.5.3.5.1-1 and A.6.5.3.5.1-2. The test consists of three successive time periods, with time durations of T1, T2, and T3 respectively.

At the start of time duration T1, the UE is in connected mode with PCell and SCell1 (cell 2) is in activated state and UE is reporting CQI for both PCell and SCell1.

Time period T2 starts when UE receives a handover command to Cell 3 that also activates SCell1 (Cell2). This is done using an RRCReconfiguration message with parameter sCellState set to activated for the SCell1 (Cell 2). The message is sent from the test equipment to the UE and is received in a subframe # denoted n at the UE antenna connector. The UE shall accomplish the activation of the SCell no later than subframe (n + Ndirect).

Time period T3 starts at (n + Ndirect), at which point UE shall be reporting a valid CQI for both PCell and SCell1.

Table A.6.5.3.5.1-1: Intra-frequency handover with direct SCell activation from FR1 to FR1 test configurations for NR PCell

| Config | Description |
| --- | --- |
| 1 | Source PCell: NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode Target PCell: NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | Source PCell: NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode Target PCell: NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | Source PCell: NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode Target PCell: NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table A.6.5.3.5.1-1A: Intra-frequency handover with direct SCell activation from FR1 to FR1 test configurations for NR SCell

| ConfigSCell | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration. |  |

Table A.6.5.3.5.1-2: General test parameters Intra-frequency handover with direct SCell activation from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | PCell |  | Cell 1 |  |
|  | SCell |  | Cell 2 |  |
|  | Target cell |  | Cell 3 |  |
| Final condition | PCell |  | Cell 3 |  |
|  | SCell |  | Cell 2 |  |
|  | neighbour cell |  | Cell 1 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH configuration index |  |  | FR1 PRACH configuration 1 | As specified in table Table 6.3.3.2-3 in TS 38.211 [6] |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 | UE is in connected mode with PCell and SCell1 (cell 2) is in activated state. UE receives a handover command |
| T2 |  | s | Ndirect | UE shall accomplish the activation of the SCell |
| T3 |  | s | 1 |  |
| THARQ |  | slot | k | k is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ feedback timing field is not present in the DCI format, the value is defined in 38.213 [3] |
| TCSI_Reporting |  | ms | 2 | the delay uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |
| k |  | ms |  | As specified in clause 4.3 of TS 38.213 [3] |

Table A.6.5.3.5.1-3: Cell specific test parameters for NR PCell for NR FR1-FR1 Intra frequency handover with direct SCell activation test case

| Parameter |  | Unit | Cell 1 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  |  | 1 |  |  | 1 |  |  |
| Duplex mode | Config 1 |  | FDD |  |  |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration | Config 1 |  | Not Applicable |  |  |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| DRx Cycle |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  |  |  |  |
| CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  |  |  |  |
| TRS configuration | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |
| OCNG Patterns |  |  | OCNG pattern 1 |  |  |  |  |  |
| SMTC Configuration |  |  | SMTC pattern 1 |  |  |  |  |  |
| SSB Configuration | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |
|  | Config 3 |  | 30 kHz |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |
|  | Config 3 |  | 30 kHz |  |  |  |  |  |
| PRACH configuration |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |
| BWP configuraiton | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |
|  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -98 |  |  |  |  |  |
| Note2 | Config 1,2 | dBm/SCS | -98 |  |  |  |  |  |
|  | Config 3 |  | -95 |  |  |  |  |  |
|  |  | dB | 8 | 8 | 8 | 8 | 8 | 8 |
|  |  | dB | 8 | 8 | 8 | 8 | 8 | 8 |
| SSB_RP | Config 1,2 | dBm/SCS | -90 | -90 | -90 | -90 | -90 | -90 |
|  | Config 3 | dBm/SCS | -87 | -87 | -87 | -87 | -87 | -87 |
| IoNote3 | Config 1,2 | dBm/ 9.36MHz | -61.41 | -57.06 | -57.06 | -57.06 | -57.06 | -61.41 |
|  | Config 3 | dBm/ 38.16MHz | -55.31 | -50.96 | -50.96 | -50.96 | -50.96 | -55.31 |
| Propagation condition |  | - | AWGN |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

Table A.6.5.3.5.1-4: Cell specific test parameters for NR SCell for NR FR1-FR1 Intra frequency handover with direct SCell activation test case

| Parameter |  | Unit | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| NR RF Channel Number |  |  | 2 |  |  |
| Duplex mode | ConfigSCell 1 |  | FDD |  |  |
|  | ConfigSCell 2,3 |  | TDD |  |  |
| TDD configuration | ConfigSCell 1 |  | Not Applicable |  |  |
|  | ConfigSCell 2 |  | TDDConf.1.1 |  |  |
|  | ConfigSCell 3 |  | TDDConf.2.1 |  |  |
| BWchannel | ConfigSCell 1 | MHz | 10: NRB,c = 52 |  |  |
|  | ConfigSCell 2 |  | 10: NRB,c = 52 |  |  |
|  | ConfigSCell 3 |  | 40: NRB,c = 106 |  |  |
| BWP BW | ConfigSCell 1 | MHz | 10: NRB,c = 52 |  |  |
|  | ConfigSCell 2 |  | 10: NRB,c = 52 |  |  |
|  | ConfigSCell 3 |  | 40: NRB,c = 106 |  |  |
| DRx Cycle |  | ms | Not Applicable |  |  |
| PDSCH Reference measurement channel | ConfigSCell 1 |  | SR.1.1 FDD |  |  |
|  | ConfigSCell 2 |  | SR.1.1 TDD |  |  |
|  | ConfigSCell 3 |  | SR.2.1 TDD |  |  |
| CORESET Reference Channel | ConfigSCell 1 |  | CR.1.1 FDD |  |  |
|  | ConfigSCell 2 |  | CR.1.1 TDD |  |  |
|  | ConfigSCell 3 |  | CR.2.1 TDD |  |  |
| TRS configuration | ConfigSCell 1 |  | TRS.1.1 FDD |  |  |
|  | ConfigSCell 2 |  | TRS.1.1 TDD |  |  |
|  | ConfigSCell 3 |  | TRS.1.2 TDD |  |  |
| OCNG Patterns |  |  | OCNG pattern 1 |  |  |
| SMTC Configuration |  |  | SMTC pattern 1 |  |  |
| SSB Configuration | ConfigSCell 1,2 |  | SSB.1 FR1 |  |  |
|  | ConfigSCell 3 |  | SSB.2 FR1 |  |  |
| PDSCH/PDCCH subcarrier spacing | ConfigSCell 1,2 | kHz | 15 kHz |  |  |
|  | ConfigSCell 3 |  | 30 kHz |  |  |
| PUCCH/PUSCH subcarrier spacing | ConfigSCell 1,2 | kHz | 15 kHz |  |  |
|  | ConfigSCell 3 |  | 30 kHz |  |  |
| PRACH configuration |  |  | FR1 PRACH configuration 1 |  |  |
| BWP configuraiton | Initial DL BWP |  | DLBWP.0.1 |  |  |
|  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -98 |  |  |
| Note2 | ConfigSCell 1,2 | dBm/SCS | -98 |  |  |
|  | ConfigSCell 3 |  | -95 |  |  |
|  |  | dB | 8 | 8 | 8 |
|  |  | dB | 8 | 8 | 8 |
| SSB_RP | ConfigSCell 1,2 | dBm/SCS | -90 | -90 | -90 |
|  | ConfigSCell 3 | dBm/SCS | -87 | -87 | -87 |
| IoNote3 | ConfigSCell 1,2 | dBm/ 9.36MHz | -61.41 | -57.06 | -61.41 |
|  | ConfigSCell 3 | dBm/ 38.16MHz | -55.31 | -50.96 | -55.31 |
| Propagation condition |  | - | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

##### A.6.5.3.5.2	Test Requirements

The UE shall be capable to transmit valid CSI report for the directly activated SCell1 no later than in subframe n+Ndirect.

The rate of correct observed SCell1 direct activation delay during repeated tests shall be at least 90%.

NOTE:	The SCell activation delay, Ndirect, can be expressed as: Ndirect = TRRC_process + Tinterrupt + T2 + T3 + Tactivation_time + TCSI_Reporting - 3ms, where:

TRRC_Process: RRC procedure delay defined in clause 12 of TS 38.331 [2],

Tinterrupt: Interruption time during handover as specified in clause 6.1.1,

T2: Delay from slot n+TRRC_Process+TinterruptNR slot length  until UE has obtained a valid TA command for the target PCell,

T3: Delay for applying the received TA for uplink transmission in the target PCell, and greater than or equal to k+1 slot, where k is defined in clause 4.2 in TS 38.213,

Tactivation_time and TCSI_Reporting are specified in clause 8.3.2, where the following definitions of TFirstSSB and TFirstSSB_MAX as defined in section 8.3.5 shall apply:

-	TFirstSSB: the time to the end of the first complete SSB burst indicated by the SMTC after slot n + (𝑇𝑅𝑅𝐶_𝑃𝑟𝑜𝑐𝑒𝑠𝑠+𝑇𝑖𝑛𝑡𝑒𝑟𝑟𝑢𝑝𝑡+𝑇2+𝑇3)/(N𝑅 𝑠𝑙𝑜𝑡 𝑙𝑒𝑛𝑔𝑡ℎ)

-	TFirstSSB_MAX: the time to the end of the first complete SSB burst indicated by the SMTC after slot n + (𝑇𝑅𝑅𝐶𝑃𝑟𝑜𝑐𝑒𝑠𝑠+𝑇𝑖𝑛𝑡𝑒𝑟𝑟𝑢𝑝𝑡+𝑇2+𝑇3)/(N𝑅 𝑠𝑙𝑜𝑡 𝑙𝑒𝑛𝑔𝑡ℎ)

This gives a total of Ndirect = 10 + 52 + TIU + T2 + T3 + Tactivation_time + TCSI_Reporting - 3 ms = 62 + 10 + 13 + 6 + 20 + 2 - 3 = 94 ms for test configurations 1 and 2.

This gives a total of Ndirect = 10 + 52 + TIU + T2 + T3 + Tactivation_time + TCSI_Reporting - 3 ms = 62 + 10 + 13 + 6 + 20 + 2 - 3 = 94 ms for test configuration 3.

During T3 the UE shall send valid CSI reports for PCell and SCell1 with non-zero CQI index and continue to send CSI reports for PCell and SCell1 (Cell 2) with non-zero CQI index until the end of T3.

All of the above test requirements shall be fulfilled in order for the observed SCell1 direct activation delay to be counted as correct.

#### A.6.5.3.4	Fast SCell Activation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle

##### A.6.5.3.4.1	Test Purpose and Environment

The purpose of this test is to verify that the fast SCell activation and deactivation times are within the requirements stated in clause 8.3.16, when the SCell in FR1 is known by the UE at the time of activation.

The supported test configurations are shown in table A.6.5.3.4.1-1 below. The test parameters are given in Tables A.6.5.3.4.1-2 and cell-specific parameters in A.6.5.3.4.1-3 below. The test consists of two successive time periods, with duration of T1and T2, respectively. There are two NR carriers, each with one cell. Both cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1, but is not aware of Cell2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2. The UE now starts monitoring the SCC. The test equipment sends a MAC message for activation of the SCell and triggering the aperiodic CSI-RS for fast SCell activation.

The point in time at which the MAC message is received at the UE antenna connector, in slot # denoted n (where n mode 20=1), defines the start of time period T2. The UE shall be able to report valid CSI in PCell for the activated SCell at latest in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length, as defined in clause 8.3. The UE shall start reporting CSI in PCell after at least one CSI-RS transmission occasion for channel measurement and reporting after slot n+THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3, where Ninterruption is the interruption length given in clause 8.2.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

Table A.6.5.3.4.1-1: known FR1 SCell activation in non-DRX for 160ms SCell measurement cycle supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table A.6.5.3.4.1-2: General test parameters for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channel (1, 2) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 7 | During this time the PSCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| THARQ | ms | Config 1: 2 Config 2: 3 Config 3: 2.5 | k1×NR slot length k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [3] that will meet the timing constraints of this test case. |
| TCSI_Reporting | ms | 15 | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |

Table A.6.5.3.4.1-3: Cell specific test parameters for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Duplex mode | Config 1 |  | FDD |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |
| TDD configuration | Config 1 |  | Not applicable |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |
| BWchannel | Config 1,2 | MHz | Note 7 |  |  |  |
|  | Config 3 |  | Note 7 |  |  |  |
| BWoccupied | Config 1,2 | RB | 52 Note 5 |  |  |  |
|  | Config 3 |  | 106 Note 6 |  |  |  |
| Initial BWP configuration |  |  | DLBWP.0.2 |  |  |  |
| TCI state |  |  | TCI.State.0 |  |  |  |
| TRS Configuration | Config 1 |  | TRS.1.1 FDD |  |  |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  | - |  |
|  | Config 2 |  | SR.1.1 TDD |  | - |  |
|  | Config 3 |  | SR.2.1 TDD |  | - |  |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD |  | - |  |
|  | Config 2 |  | CCR.1.1 TDD |  | - |  |
|  | Config 3 |  | CCR.2.1 TDD |  | - |  |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD |  | - |  |
|  | Config 2 |  | CR.1.1 TDD |  | - |  |
|  | Config 3 |  | CR.2.1 TDD |  | - |  |
| OCNG Patterns | Config 1,2 |  | OP.1 Note 5 |  |  |  |
|  | Config 3, |  | OP.1 Note 6 |  |  |  |
| SSB Configuration | Config 1,2 |  | SSB.1 FR1 |  |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |  |
| Aperiodic CSI-RS for Scell activation | Config 1 |  | N/A |  | TRS.1.3 FDD |  |
|  | Config 2 |  | N/A |  | TRS.1.3 TDD |  |
|  | Config 3 |  | N/A |  | TRS.1.4 TDD |  |
| gapBetweenBursts |  | slots | N/A |  |  |  |
| CSI-RS configuration for CSI reporting (Note 8) | Config 1 |  | CSI-RS.1.1 FDD |  |  |  |
|  | Config 2 |  | CSI-RS.1.1 TDD |  |  |  |
|  | Config 3 |  | CSI-RS.2.1 TDD |  |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  |
| reportConfigType |  |  | periodic |  |  |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |  |  |
| CSI reporting periodicity for PCell | Config 1,2 | slot | 5 |  | - |  |
|  | Config 3 |  | 10 |  | - |  |
| CSI reporting offset for PCell | Config 1,2 | slot | 3 |  | - |  |
|  | Config 3 |  | 5 |  | - |  |
| CSI reporting periodicity for SCell | Config 1,2 | slot | 5 |  | N/A |  |
|  | Config 3 |  | 10 |  | N/A |  |
| CSI reporting offset for SCell | Config 1,2 | slot | 2 |  | N/A |  |
|  | Config 3 |  | 4 |  | N/A |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 | Config 1,2 | dBm/SCS | -104 |  |  |  |
|  | Config 3 |  | -101 |  |  |  |
|  |  | dB | 17 |  |  |  |
|  |  | dB | 17 |  |  |  |
| SS-RSRPNote3 | Config 1,2 | dBm/SCS | -87 |  |  |  |
|  | Config 3 |  | -84 |  |  |  |
| CSI-RSRPNote3 | Config 1,2 | dBm/SCS |  |  | -87 |  |
|  | Config 3 |  |  |  | -84 |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -87 |  |  |  |
| Io Note3 | Config 1,2 | dBm/ 9.36MHz | -58.96 |  |  |  |
|  | Config 3 | dBm/ 38.16MHz | -52.87 |  |  |  |
| Propagation condition |  | - | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. Note 3: SS-RSRP, CSI-RSRP, Io and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. Note 8: On top of the reference configurations, CSI-RS offset should be set to meet the CSI reference resource timing definition in TS 38.214 cl. 5.2.2.5. |  |  |  |  |  |  |

##### A.6.5.3.4.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell in the first available uplink resource after at least one CSI-RS transmission occasion for channel measurement and reporting after slot (n+1+THARQ+3msNR slot length). UE is allowed to postpone CSI report to next available UL resource if an available uplink resource is subject to interruption.During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index at latest in a slot n+THARQ+Tactivtion_time+TCSI_ReportingNR slot length, Tactivation_time = TFirstATRS + 5ms, as defined in clause 8.3.16.

During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index at latest in a slot n+THARQ+Tactivtion_time+TCSI_ReportingNR slot length, Tactivation_time = TFirstATRS + 5ms, as defined in clause 8.3.16.

During T2 interruption of PCell / PSCell during SCell activation shall not happen outside the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3.16.

The interruption on any activated serving cell shall not be more than the values specified for SA in clause 8.2.2.2.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a slot THARQ+Tactivtion_time+TCSI_ReportingNR slot length as defined in clause 8.3 then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

#### A.6.5.3.5	SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle

##### A.6.5.3.5.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in clause A.6.5.3.5.1. The supported test configurations are the same as defined in clause A.6.5.3.5.1. The test parameters are the same except those described in the following clause. The listed parameter values in Tables A.6.5.3.5.1-1 will replace the values of corresponding parameters in Tables A.6.5.3.5.1-2 and the listed parameter values in Tables Table A.6.5.3.5.1-2 will replace the values of corresponding parameters in Tables A.6.5.3.5.1-3.

Table A.6.5.3.5.1-1: General test parameters for known FR1 SCell activation case, 640 ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| SCell measurement cycle (measCycleSCell) | ms | 640 |  |

Table A.6.5.3.5.1-2: Cell specific test parameters for known FR1 SCell activation case, 640ms SCell measurement cycle

| Parameter |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| gapBetweenBursts |  | slots | 2 |  |  |  |

##### A.6.5.3.5.2	Test Requirements

The test requirements defined in clause A.6.5.3.5.2 shall apply to this test case, except Tactivation_time will be replaced with the value TFirstATRS + Tgap + TATRS + 5ms.

#### A.6.5.3.6	PUCCH SCell Activation and deactivation of known SCell in FR1

##### A.6.5.3.6.1	Test Purpose and Environment

The purpose of this test is to verify that the PUCCH SCell activation and deactivation times are within the requirements stated in clause 8.3, when the PUCCH SCell in FR1 is known by the UE at the time of activation.

The supported test configurations are shown in table A.6.5.3.6.1-1 below. The test parameters are given in Tables A.6.5.3.6.1-2 and cell-specific parameters in A.6.5.3.6.1-3 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two NR carriers, each with one cell. Both cells have constant signal levels throughout the test. Cell 1 is the PCell in primary Timing Advance Group (pTAG) and cell 2 is the PUCCH SCell in the secondary Timing Advance Group (sTAG). Before the test starts the UE is connected to Cell 1, but is not aware of Cell2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

The test consists of two sub tests. In Test 1, UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment for sTAG. In Test 2, the TimeAlignmentTimer of sTAG expires before receiving the activation command.

At the beginning of T1 the UE receives an RRC message by which the PUCCH SCell (Cell 2) becomes configured on radio channel 2. The UE now starts monitoring the SCC. The test equipment sends a MAC message for activation of the PUCCH SCell.

The point in time at which the MAC message is received at the UE antenna connector, in slot # denoted n, defines the start of time period T2. In Test 1, the UE shall be able to report valid CSI on for the activated PUCCH SCell on PUCCH SCell at latest in slot n+THARQ+Tactivation_time+ X + TCSI_ReportingNR slot length, as defined in clause 8.3. In Test 2, the UE shall be able to report valid CSI on for the activated PUCCH SCell on PUCCH SCell at latest in slot n+THARQ+Tdelay_PUCCH_SCellNR slot length, as defined in clause 8.3.

Any PCell interruption due to activation of PUCCH SCell shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3, where Ninterruption is the interruption length given in clause 8.2.

Time period T3 starts when a MAC message for deactivation of PUCCH SCell, sent from the test equipment to the UE in a slot # denoted m, is received at the UE antenna connector. The UE shall carry out deactivation of the PUCCH SCell in a slot m+THARQ+3msNR slot length, as defined in clause 8.3, and The starting point of any PCell interruption due to the deactivation shall occur in the slot m+1+THARQNR slot length to m+1+THARQ+3msNR slot length, as defined in clause 8.3.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of PUCCH SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the PUCCH SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the PUCCH SCell deactivation command is sent until CQI reporting for PUCCH SCell is discontinued.

Table A.6.5.3.6.1-1: known FR1 SCell activation test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table A.6.5.3.6.1-2: General test parameters for known FR1 SCell activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channel (1, 2) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| TimeAlignmentTimer | ms | 500 | Test 1 and Test 2 |
| T1 | s | 7 | During this time the PCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| THARQ | ms | Config 1: 2 Config 2: 3 Config 3: 2.5 | k1×NR slot length k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [3] that will meet the timing constraints of this test case. |
| TCSI_Reporting | ms | 15 | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |

Table A.6.5.3.6.1-3: Cell specific test parameters for known FR1 SCell activation case

| Parameter |  | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Duplex mode | Config 1 |  | FDD |  |  |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration | Config 1 |  | Not applicable |  |  |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel | Config 1,2 | MHz | Note 7 |  |  |  |  |  |
|  | Config 3 |  | Note 7 |  |  |  |  |  |
| BWoccupied | Config 1,2 | RB | 52 Note 5 |  |  |  |  |  |
|  | Config 3 |  | 106 Note 6 |  |  |  |  |  |
| Initial BWP configuration |  |  | DLBWP.0.2 |  |  |  |  |  |
| TCI state |  |  | TCI.State.0 |  |  |  |  |  |
| TRS Configuration | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |  | - |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  | - |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  | - |  |  |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD |  |  | - |  |  |
|  | Config 2 |  | CCR.1.1 TDD |  |  | - |  |  |
|  | Config 3 |  | CCR.2.1 TDD |  |  | - |  |  |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD |  |  | - |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  | - |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  | - |  |  |
| OCNG Patterns | Config 1,2 |  | OP.1 Note 5 |  |  |  |  |  |
|  | Config 3, |  | OP.1 Note 6 |  |  |  |  |  |
| SSB Configuration | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |
| CSI-RS configuration for CSI reporting (Note 8) | Config 1 |  | CSI-RS.1.1 FDD |  |  |  |  |  |
|  | Config 2 |  | CSI-RS.1.1 TDD |  |  |  |  |  |
|  | Config 3 |  | CSI-RS.2.1 TDD |  |  |  |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  |  |  |
| reportConfigType |  |  | periodic |  |  |  |  |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |  |  |  |  |
| CSI reporting periodicity for PCell | Config 1,2 | slot | 5 |  |  | - |  |  |
|  | Config 3 |  | 10 |  |  | - |  |  |
| CSI reporting offset for PCell | Config 1,2 | slot | 3 |  |  | - |  |  |
|  | Config 3 |  | 5 |  |  | - |  |  |
| CSI reporting periodicity for SCell | Config 1,2 | slot | - |  |  | 5 |  |  |
|  | Config 3 |  | - |  |  | 10 |  |  |
| CSI reporting offset for SCell | Config 1,2 | slot | - |  |  | 2 |  |  |
|  | Config 3 |  | - |  |  | 4 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | dBm/SCS | -104 |  |  |  |  |  |
|  | Config 3 |  | -101 |  |  |  |  |  |
|  |  | dB | 17 |  |  |  |  |  |
|  |  | dB | 17 |  |  |  |  |  |
| SS-RSRPNote3 | Config 1,2 | dBm/SCS | -87 |  |  |  |  |  |
|  | Config 3 |  | -84 |  |  |  |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -87 |  |  |  |  |  |
| Io Note3 | Config 1,2 | dBm/ 9.36MHz | -58.96 |  |  |  |  |  |
|  | Config 3 | dBm/ 38.16MHz | -52.87 |  |  |  |  |  |
| Propagation condition |  | - | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. Note 3: SS-RSRP, Io and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. Note 8: On top of the reference configurations, CSI-RS offset should be set to meet the CSI reference resource timing definition in TS 38.214 cl. 5.2.2.5. |  |  |  |  |  |  |  |  |

##### A.6.5.3.6.2	Test Requirements

In Test1，during T2 the UE shall start sending CSI reports for PUCCH SCell with non-zero CQI index at latest in a slot n+THARQ+Tactivation_time+ X + TCSI_ReportingNR slot length, Tactivation_time = TFirstSSB+ 5ms, as defined in clause 8.3. In Test2，during T2 the UE shall start sending CSI reports for PUCCH SCell with non-zero CQI index at latest in a slot n+THARQ+Tdelay_PUCCH_SCellNR slot length, Tactivation_time = TFirstSSB+ 5ms, as defined in clause 8.3.

During T3 the UE shall stop sending CSI reports for PUCCH SCell at latest in a slot m+THARQ+3msNR slot length, as defined in clause 8.3.

During T2 interruption of PCell / PSCell during PUCCH SCell activation shall not happen outside the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3.

During T3 the starting point of interruption of PCell during PUCCH SCell deactivation shall not happen outside the slot m+1+THARQNR slot length to m+1+THARQ+3msNR slot length, as defined in clause 8.3.

The interruption on any activated serving cell shall not be more than the values specified for SA in clause 8.2.2.2.18.

All of the above test requirements shall be fulfilled in order for the observed PUCCH SCell activation delay and PUCCH SCell deactivation delay to be counted as correct. The rate of correct observed PUCCH SCell activation delay and PUCCH SCell deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in slot n+THARQ+Tactivation_time+ X + TCSI_ReportingNR slot length in Test 1 or in slot n+THARQ+Tdelay_PUCCH_SCellNR slot length in Test 2 as defined in clause 8.3 then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

#### A.6.5.3.7	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX

##### A.6.5.3.7.1	Test Purpose and Environment

The purpose of this test is to verify that the PUCCH SCell activation and deactivation times are within the requirements stated in clause 8.3, when the PUCCH SCell in FR1 is unknown by the UE at the time of activation. In this test, UE shall support cross PUCCH group CSI reporting capability [TBD].

The supported test configurations are shown in Table A.6.5.3.7.1-1 below. The test parameters are given in Table A.6.5.3.7.1-2 and cell-specific parameters in Table A.6.5.3.7.1-3 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two NR carriers, each with one cell. Cell 1 is the PCell in primary Timing Advance Group (pTAG) and Cell 2 is the PUCCH SCell in the secondary Timing Advance Group (sTAG). Both cells have constant signal levels throughout the test.

There are two sub tests in this section.

For Test 1 (valid TA case), UE is provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment for sTAG.

For Test 2 (invalid TA case), TimeAlignmentTimer of sTAG expires before UE receives the activation command

Before the test starts the UE is connected to PCell, but is not aware of PUCCH SCell. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the PUCCH SCell becomes configured on NR. During T1 the PUCCH SCell is powered off and UE is not aware of PUCCH SCell.

The point in time at which the MAC message is received at the UE antenna connector, in slot # denoted n, defines the start of time period T2. The UE shall be able to report valid CSI on PUCCH SCell for the activated PUCCH SCell at latest in

slot n+THARQ+Tactivation_time+X+TCSI_ReportingNR slot length for Test 1, or

slot n+THARQ+Tdelay_PUCCH_SCellNR slot length  for Test 2

Note: Tdelay_PUCCH_SCell = Tactivation_time + [X] + max ((TFirst_available_CSI + TCSI_processing), (T1+T2+T3)) + TCSI_reporting_after, as defined in clause 8.3,

Any PCell interruption due to activation of PUCCH SCell shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+TXNR slot length+Ninterruption, as defined in clause 8.3, where Ninterruption is the interruption length given in clause 8.2, and TX is Tactivation_time+X+TCSI_Reporting and Tdelay_PUCCH_SCell for Test 1 and Test 2, respectively.

Time period T3 starts when a MAC message for deactivation of PUCCH SCell, sent from the test equipment to the UE in a slot # denoted m, is received at the UE antenna connector. The UE shall carry out deactivation of the PUCCH SCell in a slot m+THARQ+3msNR slot length, as defined in clause 8.3, and the starting point of any PCell interruption due to the deactivation shall occur in the slot m+1+THARQNR slot length to m+1+THARQ+3msNR slot length, as defined in clause 8.3.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of PUCCH SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the PUCCH SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the PUCCH SCell deactivation command is sent until CQI reporting for PUCCH SCell is discontinued.

Table A.6.5.3.7.1-1: unknown FR1 PUCCH SCell activation in non-DRX for 160ms SCell measurement cycle supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table A.6.5.3.7.1-2: General test parameters for unknown FR1 PUCCH SCell activation case, 160ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channel (1, 2) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated PUCCH secondary cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| TimeAlighmentTimer | ms | 500 | Identify the whether TA information is valie or not |
| T1 | s | [7] | During this time the PCell shall be known and the PUCCH SCell configured and detected. |
| T2 | s | [1] | During this time the UE shall activate the PUCCH SCell. |
| T3 | s | [1] | During this time the UE shall deactivate the PUCCH SCell. |
| THARQ | ms | Config 1: 2 Config 2: 3 Config 3: 2.5 | k1×NR slot length k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [3] that will meet the timing constraints of this test case. |
| TCSI_Reporting | ms | 15 | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |

Table A.6.5.3.7.1-3: Cell specific test parameters for unknown FR1 PUCCH SCell activation case, 160ms SCell measurement cycle

| Parameter |  | Unit | Cell 1 |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 |  | T3 |  |
| Duplex mode | Config 1 |  | FDD |  |  |  |  |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |
| TDD configuration | Config 1 |  | Not applicable |  |  |  |  |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |
| BWchannel | Config 1,2 | MHz | Note 7 |  |  |  |  |  |  |  |
|  | Config 3 |  | Note 7 |  |  |  |  |  |  |  |
| BWoccupied | Config 1,2 | RB | 52 Note 5 |  |  |  |  |  |  |  |
|  | Config 3 |  | 106 Note 6 |  |  |  |  |  |  |  |
| Initial BWP configuration |  |  | DLBWP.0.2 |  |  |  |  |  |  |  |
| TCI state |  |  | TCI.State.0 |  |  |  |  |  |  |  |
| TRS Configuration | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |  |  |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |  |  |
|  | Config 2 |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |  |  |
|  | Config 3 |  | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |  |  |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD |  |  | N/A |  |  |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  | N/A |  |  |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  | N/A |  |  |  |  |
| OCNG Patterns | Config 1,2 |  | OP.1 Note 5 |  |  |  |  |  |  |  |
|  | Config 3, |  | OP.1 Note 6 |  |  |  |  |  |  |  |
| SSB Configuration | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |
| CSI-RS configuration for CSI reporting (Note 8) | Config 1 |  | CSI-RS.1.1 FDD |  |  |  |  |  |  |  |
|  | Config 2 |  | CSI-RS.1.1 TDD |  |  |  |  |  |  |  |
|  | Config 3 |  | CSI-RS.2.1 TDD |  |  |  |  |  |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  |  |  |  |  |
| reportConfigType |  |  | periodic |  |  |  |  |  |  |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |  |  |  |  |  |  |
| CSI reporting (CQI index non 0) periodicity for PUCCH SCell | Config 1,2 | slot | N/A |  |  | 5 |  |  |  |  |
|  | Config 3 |  | N/A |  |  | 10 |  |  |  |  |
| CSI reporting (CQI index non 0) offset for PUCCH SCell | Config 1,2 | slot | N/A |  |  | 4 |  |  |  |  |
|  | Config 3 |  | N/A |  |  | 6 |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | dBm/SCS | -104 |  |  | N/A |  | -104 |  | -104 |
|  | Config 3 |  | -101 |  |  | N/A |  | -101 |  | -101 |
|  |  | dB | 17 |  |  | -infinity |  | 17 |  | 17 |
|  |  | dB | 17 |  |  | -infinity |  | 17 |  | 17 |
| SS-RSRPNote3 | Config 1,2 | dBm/SCS | -87 |  |  | -infinity |  | -87 |  | -87 |
|  | Config 3 |  | -84 |  |  | -infinity |  | -84 |  | -84 |
| SCH_RP Note 3 |  | dBm/15 kHz | -87 |  |  | N/A |  | -87 |  | -87 |
| Io Note3 | Config 1,2 | dBm/ 9.36MHz | -58.96 |  |  | N/A |  | -58.96 |  | -58.96 |
|  | Config 3 | dBm/ 38.16MHz | -52.87 |  |  | N/A |  | -52.87 |  | -52.87 |
| Propagation condition |  | - | AWGN |  |  | N/A |  | AWGN |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. Note 3: SS-RSRP, Io and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. Note 8: On top of the reference configurations, CSI-RS offset should be set to meet the CSI reference resource timing definition in TS 38.214 cl. 5.2.2.5. |  |  |  |  |  |  |  |  |  |  |

##### A.6.5.3.7.2	Test Requirements

During T2, as defined in clause 8.3, the UE shall start sending CSI reports for PUCCH SCell on PUCCH SCell with non-zero CQI index at latest in

a slot n+THARQ+Tactivtion_time+TCSI_ReportingNR slot length for Test 1.

a slot n+THARQ+Tdelay_PUCCH_SCellNR slot length  for Test 2.

During T3 the UE shall stop sending CSI reports for PUCCH SCell at latest in a slot m+THARQ+3msNR slot length, as defined in clause 8.3.

During T2 interruption of PCell during PUCCH SCell activation shall not happen outside the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3.

During T3 the starting point of interruption of PCell during PUCCH SCell deactivation shall not happen outside the slot m+1+THARQNR slot length to m+1+THARQ+3msNR slot length, as defined in clause 8.3.

The interruption on any activated serving cell shall not be more than the values specified for SA in clause 8.2.2.2.18.

All of the above test requirements shall be fulfilled in order for the observed PUCCH SCell activation delay and PUCCH SCell deactivation delay to be counted as correct. The rate of correct observed PUCCH SCell activation delay and PUCCH SCell deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a slot  n+THARQ+Tactivtion_time+TCSI_ReportingNR slot length  and in a slot n+THARQ+Tdelay_PUCCH_SCellNR slot length for Test 1 and Test 2, respectively, as defined in clause 8.3 then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

#### A.6.5.3.8	SCell Activation and Deactivation of one FR1 known PUCCH SCell and one FR1 unknown SCell with single activation/deactivation command

##### A.6.5.3.8.1	Test Purpose and Environment

The purpose of this test is to verify the SCell activation and deactivation delay requirements for PUCCH SCell with multiple SCells specified in clause 8.3.13 and 8.3.15, when one configured deactivated known PUCCH SCells in FR1 and one configured unknown SCell in FR1 by the UE at the time of activation.

The supported test configurations are defined in table A.6.5.3.8.1-1 below. The test parameters are given in table A.6.5.3.8.1-2 and cell-specific parameters in A.6.5.3.8.1-3 below.

The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are three NR carriers. All Cells has constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCC), but is not aware of Cell 2 (PUCCH SCell) and Cell 3(SCell). The UE is monitoring the Cell 1 (PCC). The UE shall be continuously scheduled in the Cell 1 throughout the whole test. PCC, SCC of Cell 2 and SCC of Cell3 are on different FR1 bands.

At the beginning of T1 the UE receives an RRC message by which the PUCCH SCell (Cell 2) and SCell (Cell 3) become configured on radio channel 2 and 3 respectively.

A MAC message for activation of UCCH SCell (Cell 2) and SCell (Cell 3) is sent by the test equipment 100ms after the RRC message, in a slot # denoted m. The point in time at which the MAC message for activation of UCCH SCell (Cell 2) and SCell (Cell 3) is received at the UE antenna connector defines the start of time period T2. Immediately at beginning of T2 the transmission power of cell 2 and cell 3 are increased to same level as for cell 1. The UE shall be able to report valid CSI on PCell for the activated PUCCH SCell (Cell2) at latest in slot m+THARQ+Tdelay_multiple_SCells_other_SCellNR slot length  as defined in clause 8.3.13 provided the PUCCH SCell can be successfully detected on the first attempt.

For Cell2 activtion, the UE shall start reporting CSI in PCell in slot n+THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the PUCCH SCell activation has been completed. For Cell3 activtion, the UE shall start reporting CSI in PCell in slot n+THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the DL SCell activation has been completed.

Any PCell interruption due to activation of PUCCH SCell shall occur in the slot m+1+THARQNR slot length to slot m+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3, where  Ninterruption is the interruption length given in section 8.2.

Time period T3 starts when a MAC message for deactivation of the SCells (Cell 2 and Cell 3), sent from the test equipment to the UE in a slot # denoted n, is received at the UE antenna connector. The UE shall carry out deactivation of the PUCCH SCell at latest in slot n+THARQ+3msNR slot length  as defined in clause 8.3. The starting point of PCell interruption due to the deactivation shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3msNR slot length, as defined in clause 8.3.

The test equipment verifies the PUCCH SCell activation time by counting the slots from the time when the PUCCH SCell activation command is sent until a CSI report with other than CQI index 0 is received. The test equipment verifies the DL SCell activation time by counting the slots from the time when the DL SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the PUCCH SCell deactivation time by counting the slots from the time when the SCell deactivation command is sent until CQI reporting for PUCCH SCell is discontinued. The test equipment verifies the DL SCell deactivation time by counting the slots from the time when the SCell deactivation command is sent until CQI reporting for DL SCell is discontinued.

Table A.6.5.3.8.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table A.6.5.3.8.1-2: General test parameters for unknown FR1 SCell activation case with 2 deactivated SCells, 160ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2,3 | Three NR radio channel (1, 2, 3) are used for this test |
| Active Cell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated PUCCH SCell |  | Cell 2 | Configured deactivated PUCCH SCell on NR RF channel number 2 which is an inter-band CC to PCC of Cell 1; |
| Configured deactivated SCell |  | Cell 3 | Configured deactivated SCell on NR RF channel number 3 which is an inter-band CC to PCC of Cell 1 and SCC of Cell2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell2 timing offset to cell1 | s | 0 |  |
| Cell3 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| Time alignment error between cell3 and cell1 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| T1 | ms | 100 | During this time the PSCell shall be known and the SCell configured, but not detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| THARQ | ms | k1NR slot length | k1 is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ feedback timing field is not present in the DCI format, the value is defined in 38.213 [3] |
| TCSI_Reporting | ms | 15 | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |
| k | slot |  | As specified in clause 4.3 of TS 38.213 [3] |

Table A.6.5.3.8.1-3: Cell specific test parameters for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter |  |  |  | Unit | Cell 3 |  | Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T2 | T3 | T2 | T3 |
| SSB ARFCN |  |  |  |  | Freq2 |  | Freq3 |  |
| Duplex mode |  | Config 1,4 |  |  | FDD |  |  |  |
|  |  | Config 2,3,5,6 |  |  | TDD |  |  |  |
| TDD configuration |  | Config 1,4 |  |  | Not Applicable |  |  |  |
|  |  | Config 2,5 |  |  | TDDConf.1.1 |  |  |  |
|  |  | Config 3,6 |  |  | TDDConf.2.1 |  |  |  |
| BWchannel |  | Config 1,4 |  | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 2,5 |  |  | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3,6 |  |  | 40: NRB,c = 106 |  |  |  |
| DL initial BWP configuration |  | Config 1, 2, 3, 4, 5, 6 |  |  | DLBWP.0.1 |  |  |  |
| DL dedicated BWP configuration |  | Config 1, 2, 3, 4, 5, 6 |  |  | DLBWP.1.1 |  |  |  |
| UL initial BWP configuration |  | Config 1, 2, 3, 4, 5, 6 |  |  | ULBWP.0.1 |  |  |  |
| UL dedicated BWP configuration |  | Config 1, 2, 3, 4, 5, 6 |  |  | ULBWP.1.1 |  |  |  |
| DRX Cycle |  |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  |  | SR.1.1 FDD |  | SR.1.1 FDD |  |
|  |  | Config 2,5 |  |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  | Config 3,6 |  |  | SR.2.1 TDD |  | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  |  | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | Config 2,5 |  |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | Config 3,6 |  |  | CR.2.1 TDD |  | CR.2.1 TDD |  |
| RMC CORESET Reference Channel |  | Config 1,4 |  |  | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
|  |  | Config 2,5 |  |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | Config 3,6 |  |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| TRS configuration |  | Config 1,4 |  |  | TRS.1.1 FDD |  | TRS.1.1 FDD |  |
|  |  | Config 2,5 |  |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |
|  |  | Config 3,6 |  |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |  |
| SMTC configuration |  |  |  |  | SMTC.1 |  |  |  |
| SSB configuration |  | Config 1,2,4,5 |  |  | SSB.1 FR1 |  |  |  |
|  |  | Config 3,6 |  |  | SSB.2 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4,5 |  | kHz | 15 kHz |  |  |  |
|  |  | Config 3,6 |  |  | 30kHz |  |  |  |
| CSI-RS configuration for CSI reporting |  | Config 1,4 |  |  | CSI-RS.1.1 FDD |  |  |  |
|  |  | Config 2,5 |  |  | CSI-RS.1.1 TDD |  |  |  |
|  |  | Config 3,6 |  |  | CSI-RS.2.1 TDD |  |  |  |
| reportConfigType |  | Config 1-6 |  |  | periodic |  |  |  |
| reportQuantity |  | Config 1-6 |  |  | cri-RI-PMI-CQI |  |  |  |
| CSI reporting periodicity |  | Config 1,2,4,5 |  | slot | 5 |  | N/A |  |
| CSI reporting offset |  | Config 3,6 |  | slot | 10 |  | N/A |  |
|  |  | Config 1,2,4,5 |  |  | 2 |  | N/A |  |
|  |  | Config 3,6 |  |  | 4 |  | N/A |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  |  |  | dBm/15kHz | -104 |  |  |  |
| Note2 |  |  | Config 1,2,4,5 | dBm/SCS | -104 |  |  |  |
|  |  |  | Config 3,6 |  | -101 |  |  |  |
|  |  |  |  | dB | 17 |  |  |  |
|  |  |  |  | dB | 17 |  |  |  |
| SS-RSRPNote3 |  |  | Config 1,2,4,5 | dBm/SCS | -87 |  |  |  |
|  |  |  | Config 3,6 |  | -84 |  |  |  |
| SCH_RP Note 3 |  |  |  | dBm/15 kHz | -87 |  |  |  |
| Propagation condition |  |  |  | - | AWGN |  |  |  |
| IoNote3 | Config 1,2,4,5 |  |  | dBm/ 9.36MHz | -58.96 |  |  |  |
|  | Config 3,6 |  |  | dBm/ 38.16MHz | -52.87 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. Note 3: SS-RSRP, Io and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2.] Note 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |  |  |  |  |  |

##### A.6.5.3.8.2	Test Requirements

During T2, the UE shall send the first CSI report for SCell in the first available uplink resource after slot (m+k). UE is allowed to postpone CSI report to next available uplink resource if an available uplink resource is subject to interruption.  Whether CSI report in slot (m+k) was interrupted is checked by monitoring ACK/NACK sent in PCell in slot (m+k). And the UE shall be able to report valid CSI for the activated PUCCH SCell (Cell2) at latest in slot m+THARQ+Tdelay_multiple_SCells_other_SCellNR slot length  as defined in clause 8.3.13. And the PCell interruption due to activation of PUCCH SCell shall occur in the slot m+1+THARQNR slot length to slot m+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3, where Ninterruption is the interruption length given in section 8.2.

During T3, the UE shall carry out deactivation of the PUCCH SCell at latest in slot n+THARQ+3msNR slot length  as defined in clause 8.3. And the starting point of PCell interruption due to the deactivation shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3msNR slot length, as defined in clause 8.3.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay and SCell deactivation delay during repeated tests shall be at least 90%.

#### A.6.5.3.9	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR1 in non-DRX

##### A.6.5.3.9.1	Test Purpose and Environment

The purpose of this test is to verify that the PUCCH SCell and DL SCell activation and deactivation times are within the requirements stated in clause 8.3.13, when the PUCCH SCell in FR1 and DL SCell in FR1 is unknown to the UE at the time of activation.

The supported test configurations are shown in table A.6.5.3.9.1-1 below. The test parameters are given in Tables A.6.5.3.9.1-2 and cell-specific parameters in A.6.5.3.9.1-3 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are three NR carriers, each with one cell. All cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1, but is not aware of Cell2 (PUCCH SCell) and Cell3(DL SCell). The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test. PCC, SCC of Cell 2 and SCC of Cell 3 are on different bands. The primary PUCCH group contain Cell1 and the secondary PUCCH group contains Cell2  and Cell3.

At the beginning of T1 the UE receives an RRC message by which the PUCCH SCell (Cell 2) and DL SCell (Cell 3) becomes configured on radio channel 2 and 3 respectively. The UE starts monitoring the SCC1(Cell 2 CC) and SCC2(Cell 3 CC). The test equipment sends a MAC message for activation of the PUCCH SCell and DL SCell simultaneously.

The point in time at which the MAC message is received at the UE antenna connector, in slot # denoted n, defines the start of time period T2. The UE shall be able to report valid CSI in PCell for the activated PUCCH SCell at latest in slot n+ THARQ+Tdelay_multiple_SCells_PUCCH_SCellNR slot length, and report valid CSI in PCell for the activated DL SCell at latest in slot n+THARQ+Tdelay_multiple_SCells_other_SCellNR slot length, as defined in clause 8.3.13. In this test case, both valid TA and invalid TA cases shall be tested.

Test for case when UE has valid TA: the TimeAlignmentTimer [2] associated with the TAG containing the PUCCH SCell is running, and Tdelay_multiple_SCells_PUCCH_SCell = Tactivation_time_multiple_scells + [X]*Ttarget_PL_RS + TCSI_Reporting .

Test for case when UE do not have valid TA: Tdelay_multiple_SCells_PUCCH_SCell = Tactivation_time_multiple_scells + max ((TFirst_available_CSI + TCSI_processing), [X]*Ttarget_PL_RS, (T1+T2+T3)) + TCSI_reporting_after

Tactivation_time_multiple_scells is the target SCell activation delay in millisecond in multiple SCell activation scenario as specified in section 8.3.7

For Cell2 activtion, the UE shall start reporting CSI in PCell in slot n+THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the PUCCH SCell activation has been completed. For Cell3 activtion, the UE shall start reporting CSI in PCell in slot n+THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the DL SCell activation has been completed.

Any PCell interruption due to activation of PUCCH SCell or DL SCell shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3.13, where Ninterruption is the interruption length given in clause 8.2.2.2.7.

Time period T3 starts when a MAC message for deactivation of PUCCH SCell abd DL SCell, sent from the test equipment to the UE in a slot # denoted m, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell in a slot m+THARQ+3msNR slot length, as defined in clause 8.3.15, and the starting point of any PCell interruption due to the deactivation shall occur in the slot m+1+THARQNR slot length to m+1+THARQ+3msNR slot length, as defined in clause 8.3.15.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of PUCCH SCell and DL SCell, respectively.

The test equipment verifies the PUCCH SCell activation time by counting the slots from the time when the PUCCH SCell activation command is sent until a CSI report with other than CQI index 0 is received. The test equipment verifies the DL SCell activation time by counting the slots from the time when the DL SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the PUCCH SCell deactivation time by counting the slots from the time when the SCell deactivation command is sent until CQI reporting for PUCCH SCell is discontinued. The test equipment verifies the DL SCell deactivation time by counting the slots from the time when the SCell deactivation command is sent until CQI reporting for DL SCell is discontinued.

Table A.6.5.3.9.1-1: unknown FR1 PUCCH SCell and DL SCell activation test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode, for both PUCCH SCell and DL SCell |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode, for both PUCCH SCell and DL |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode, for both PUCCH SCell and DL |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration |  |

Table A.6.5.3.9.1-2: General test parameters for unknown FR1 PUCCH SCell and DL SCell activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2,3 | Three NR radio channel (1, 2, 3) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated PUCCH SCell |  | Cell 2 | Configured deactivated PUCCH secondary cell on NR RF channel number 2 |
| Configured deactivated DL SCell |  | Cell 3 | Configured deactivated DL secondary cell on NR RF channel number 3 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 | For both cell 2 and cell 3 |
| Cell2/Cell 3 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1; cell3 and cell1; cell3 and cell2 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 0.1 | During this time the PCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| THARQ | ms | Config 1: 2 Config 2: 3 Config 3: 2.5 | k1×NR slot length k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [3] that will meet the timing constraints of this test case. |
| TCSI_Reporting | ms | 15 | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |

Table A.6.5.3.9.1-3: Cell specific test parameters for unknown FR1 PUCCH SCell and DL SCell activation case

| Parameter |  | Unit | Cell 1 |  |  | Cell 2 and Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Duplex mode | Config 1 |  | FDD |  |  |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration | Config 1 |  | Not applicable |  |  |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel | Config 1,2 | MHz | Note 7 |  |  |  |  |  |
|  | Config 3 |  | Note 7 |  |  |  |  |  |
| BWoccupied | Config 1,2 | RB | 52 Note 5 |  |  |  |  |  |
|  | Config 3 |  | 106 Note 6 |  |  |  |  |  |
| Initial BWP configuration |  |  | DLBWP.0.2 |  |  |  |  |  |
| TCI state |  |  | TCI.State.0 |  |  |  |  |  |
| TRS Configuration | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |  | - |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  | - |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  | - |  |  |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD |  |  | - |  |  |
|  | Config 2 |  | CCR.1.1 TDD |  |  | - |  |  |
|  | Config 3 |  | CCR.2.1 TDD |  |  | - |  |  |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD |  |  | - |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  | - |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  | - |  |  |
| OCNG Patterns | Config 1,2 |  | OP.1 Note 5 |  |  |  |  |  |
|  | Config 3, |  | OP.1 Note 6 |  |  |  |  |  |
| SSB Configuration | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |
| CSI-RS configuration for CSI reporting (Note 8) | Config 1 |  | CSI-RS.1.1 FDD |  |  |  |  |  |
|  | Config 2 |  | CSI-RS.1.1 TDD |  |  |  |  |  |
|  | Config 3 |  | CSI-RS.2.1 TDD |  |  |  |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  |  |  |
| reportConfigType |  |  | periodic |  |  |  |  |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |  |  |  |  |
| CSI reporting periodicity for PCell | Config 1,2 | slot | 5 |  |  | - |  |  |
|  | Config 3 |  | 10 |  |  | - |  |  |
| CSI reporting offset for PCell | Config 1,2 | slot | 3 |  |  | - |  |  |
|  | Config 3 |  | 5 |  |  | - |  |  |
| CSI reporting periodicity for SCell | Config 1,2 | slot | 5 |  |  | 5 (on cell 2) |  |  |
|  | Config 3 |  | 10 |  |  | 10 (on cell 2) |  |  |
| CSI reporting offset for SCell | Config 1,2 | slot | 2 |  |  | 2 (on cell 2) |  |  |
|  | Config 3 |  | 4 |  |  | 4 (on cell 2) |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | dBm/SCS | -104 |  |  |  |  |  |
|  | Config 3 |  | -101 |  |  |  |  |  |
|  |  | dB | 17 |  |  |  |  |  |
|  |  | dB | 17 |  |  |  |  |  |
| SS-RSRPNote3 | Config 1,2 | dBm/SCS | -87 |  |  |  |  |  |
|  | Config 3 |  | -84 |  |  |  |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -87 |  |  |  |  |  |
| Io Note3 | Config 1,2 | dBm/ 9.36MHz | -58.96 |  |  |  |  |  |
|  | Config 3 | dBm/ 38.16MHz | -52.87 |  |  |  |  |  |
| Propagation condition |  | - | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. Note 3: SS-RSRP, Io and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. Note 8: On top of the reference configurations, CSI-RS offset should be set to meet the CSI reference resource timing definition in TS 38.214 cl. 5.2.2.5. |  |  |  |  |  |  |  |  |

##### A.6.5.3.9.2	Test Requirements

The test requirements defined in clause A.6.5.3.9.2 shall apply to this test case for PUCCH SCell, except Tactivation_time will be replaced with the value [TBD] as defined in clause 8.3.13.

The test requirements defined in clause A.6.5.3.3.2 shall apply to this test case for DL SCell, except Tactivation_time will be replaced with the value [TBD] as defined in clause 8.3.13.

### A.6.5.4	UE UL carrier RRC reconfiguration Delay

#### A.6.5.4.1	UE UL carrier RRC reconfiguration Delay

Table A.6.5.4.1-1 - Table A.6.5.4.1-4 : Void

##### A.6.5.4.1.1	Test Purpose and Environment

The purpose of this test is to verify that when the UE receives a RRC message implying NR UL or Supplementary UL carrier configuration, the UE shall be ready to start transmission on the newly configured carrier within the time limits specified in clause 8.4.2 and 8.4.3 for configuring and deconfiguring, respectively.

There are two cells: FR1 PCell (cell 1) and FR1 SCell (cell 2). Both NR uplink and supplementary uplink are broadcast by ServingCellConfigCommonSIB. The test parameters for PCell and SCell are given in Table A. 6.5.4.1.1-1, Table A.6.5.4.1.1-2, Table A.6.5.4.1.1-3 and Table A.6.5.4.1.1-4 below.  In test 1, the test consists of three time periods, with duration of T1, T2 and T3 respectively. During time duration T1, NR uplink of cell 2 is configured to UE. At the start of T2, a supplementary uplink of cell 2 is configured to UE through RRCReconfiguration, then UE shall start transmission both on the NR uplink and supplementary uplink. At the start of T3, the supplementary uplink is released through RRCReconfiguration.

In test 2, the test consists of three time periods, with duration of T1, T2 and T3 respectively. During time duration T1, supplementray uplink on cell 2 is configured to UE. At the start of T2, a NR uplink is configured to UE through RRCReconfiguration, then UE shall start transmission both on the NR uplink and supplementary uplink. At the start of T3, the NR uplink is released through RRCReconfiguration.

Table A.6.5.4.1.1-1: Supported test configurations

| Configuration | PCell (Cell 1) | SCell (Cell 2) |
| --- | --- | --- |
| 1 | 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode | DL and UL: 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode; SUL: 15 kHz SCS, ≥10 MHz bandwidth, SUL duplex mode |
| 2 | 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode | DL and UL: 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode; SUL: 15 kHz SCS, ≥10 MHz bandwidth, SUL duplex mode |
| 3 | 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode | DL and UL: 30kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode; SUL: 30kHz SCS, ≥40 MHz bandwidth, SUL duplex mode |
| 4 | 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode | DL and UL: 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode; SUL: 15 kHz SCS, ≥10 MHz bandwidth, SUL duplex mode |
| 5 | 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode | DL and UL: 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode; SUL: 15 kHz SCS, ≥10 MHz bandwidth, SUL duplex mode |
| 6 | 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode | DL and UL: 30kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode; SUL: 30kHz SCS, ≥40 MHz bandwidth, SUL duplex mode |
| 7 | 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode | DL and UL: 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode; SUL: 15 kHz SCS, ≥10 MHz bandwidth, SUL duplex mode |
| 8 | 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode | DL and UL: 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode; SUL: 15 kHz SCS, ≥10 MHz bandwidth, SUL duplex mode |
| 9 | 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode | DL and UL: 30kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode; SUL: 30kHz SCS, ≥40 MHz bandwidth, SUL duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |  |

Table A.6.5.4.1.1-2: General test parameters for NR standalone UE UL carrier RRC reconfiguration Delay on Pcell

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | Config 1,2,3, 4, 5, 6, 7, 8, 9 | 1, 2 | Two radio channels are used for these two tests. |
| Active cell |  | Config 1,2,3, 4, 5, 6, 7, 8, 9 | Cell 1: FR1 PCell Cell 2: FR1 SCell | PCell on RF channel number 1 FR1 SCell on RF channel number 2 |
| CP length |  | Config 1,2,3, 4, 5, 6, 7, 8, 9 | Normal |  |
| DRX |  | Config 1,2,3, 4, 5, 6, 7, 8, 9 | OFF |  |
| Measurement gap pattern Id |  | Config 1,2,3, 4, 5, 6, 7, 8, 9 | OFF |  |
| Filter coefficient |  | Config 1,2,3, 4, 5, 6, 7, 8, 9 | 0 | L3 filtering is not used |
| T1 | s | Config 1,2,3, 4, 5, 6, 7, 8, 9 | 5 |  |
| T2 | s | Config 1,2,3, 4, 5, 6, 7, 8, 9 | 5 |  |
| T3 | s | Config 1,2,3, 4, 5, 6, 7, 8, 9 | 5 |  |

Table A.6.5.4.1.1-3: NR Cell specific test parameters for NR standalone UE UL carrier RRC reconfiguration Delay on PCell (Cell 1)

| Parameter | Unit | Test Configuration | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Channel number |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 1 |  |  | 1 |  |  |
| TDD configuration |  | Conf 1, 2, 3 | N/A |  |  | N/A |  |  |
|  |  | Conf 4, 5, 6 | TDD Conf.1.1 |  |  | TDD Conf.1.1 |  |  |
|  |  | Conf 7, 8, 9 | TDD Conf.2.1 |  |  | TDD Conf.2.1 |  |  |
| BWchannel | MHz | Conf 1, 2, 3 | Note 6 |  |  | Note 6 |  |  |
|  |  | Conf 4, 5, 6 | Note 6 |  |  | Note 6 |  |  |
|  |  | Conf 7, 8, 9 | Note 6 |  |  | Note 6 |  |  |
| BWoccupied | RB | Conf 1, 2, 3 | 52 Note 4 |  |  | 52 Note 4 |  |  |
|  |  | Conf 4, 5, 6 | 52 Note 4 |  |  | 52 Note 4 |  |  |
|  |  | Conf 7, 8, 9 | 106 Note 5 |  |  | 106 Note 5 |  |  |
| PDSCH reference measurement channel as defined in A.3.1.1 |  | Conf 1, 2, 3 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
|  |  | Conf 4, 5, 6 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | Conf 7, 8, 9 | SR 2.1 TDD |  |  | SR 2.1 TDD |  |  |
| RMSI CORESET reference measurement channel as defined in A.3.1.2 |  | Conf 1, 2, 3 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
|  |  | Conf 4, 5, 6 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | Conf 7, 8, 9 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| RMC CORESET reference measurement channel as defined in A.3.1.3 |  | Conf 1, 2, 3 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
|  |  | Conf 4, 5, 6 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | Conf 7, 8, 9 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern Note 1 |  | Conf 1, 2, 3, 4, 5, 6 | OP.1 Note 4 |  |  | OP.1 Note 4 |  |  |
|  |  | Config 7, 8, 9 | OP.1 Note 5 |  |  | OP.1 Note 5 |  |  |
| SSB configuration |  | Conf 1, 2, 3, 4, 5, 6 | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |
|  |  | Conf 7, 8, 9 | SSB.2 FR1 |  |  | SSB.2 FR1 |  |  |
| SMTC configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | SMTC.1 |  |  | SMTC.1 |  |  |
| CSI-RS for tracking |  | Conf 1 | TRS.1.1 FDD |  |  | TRS.1.1 FDD |  |  |
|  |  | Conf 2 | TRS.1.1 FDD |  |  | TRS.1.1 FDD |  |  |
|  |  | Conf 3 | TRS.1.1 FDD |  |  | TRS.1.1 FDD |  |  |
|  |  | Conf 4 | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | Conf 5 | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | Conf 6 | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | Conf 7 | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
|  |  | Conf 8 | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
|  |  | Conf 9 | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| DL initial BWP configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| DL dedicated BWP configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | DLBWP.1.1 |  |  | DLBWP.1.1 |  |  |
| UL dedicated BWP configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | ULBWP.1.1 |  |  | ULBWP.1.1 |  |  |
| EPRE ratio of PSS to SSS | dB | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 0 |  |  | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |  |  |
| Note 2 | dBm / 15kHz | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | -102 |  |  | -102 |  |  |
|  | dBm/ SCS | Conf 1,2,3,4,5,6 | -102 |  |  | -102 |  |  |
|  |  | Conf 7,8,9 | -99 |  |  | -99 |  |  |
|  | dB | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 16 | 16 | 16 | 16 | 16 | 16 |
| Note 3 | dB | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 16 | 16 | 16 | 16 | 16 | 16 |
| SS-RSRP Note 3 | dBm/ SCS | Conf 1,2,3,4,5,6 | -86 | -86 | -86 | -86 | -86 | -86 |
|  |  | Conf 7,8,9 | -83 | -83 | -83 | -83 | -83 | -83 |
| Io Note 3 | dBm/ 9.36 MHz | Conf 1,2,3,4,5,6 | -57.9 | -57.9 | -57.9 | -57.9 | -57.9 | -57.9 |
|  | dBm/ 38.16MHz | Conf 7,8,9 | -51.8 | -51.8 | -51.8 | -51.8 | -51.8 | -51.8 |
| Propagation Condition |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | AWGN |  |  | AWGN |  |  |
| Antenna configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 1 x 2 |  |  | 1 x 2 |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. NOTE 3: , Io, and SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 6: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |  |  |  |  |  |

Table A.6.5.4.1.1-4: NR Cell specific test parameters for NR standalone UE UL carrier RRC reconfiguration Delay on SCell (Cell 2)

| Parameter | Unit | Test Configuration | Test 1 |  |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |  | T3 | T1 | T2 | T3 |
| Channel number |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 2 |  |  |  | 2 |  |  |
| TDD configuration |  | Conf 1, 4, 7 | N/A |  |  |  | N/A |  |  |
|  |  | Conf 2, 5, 8 | TDDConf.1.1 |  |  |  | TDDConf.1.1 |  |  |
|  |  | Conf 3, 6, 9 | TDDConf.2.1 |  |  |  | TDDConf.2.1 |  |  |
| BWchannel | MHz | Conf 1, 4, 7 | Note 6 |  |  |  | Note 6 |  |  |
|  |  | Conf 2, 5, 8 | Note 6 |  |  |  | Note 6 |  |  |
|  |  | Conf 3, 6, 9 | Note 6 |  |  |  | Note 6 |  |  |
| BWoccupied | RB | Conf 1, 4, 7 | 52 Note 4 |  |  |  | 52 Note 4 |  |  |
|  |  | Conf 2, 5, 8 | 52 Note 4 |  |  |  | 52 Note 4 |  |  |
|  |  | Conf 3, 6, 9 | 106 Note 5 |  |  |  | 106 Note 5 |  |  |
| PUSCH parameters for NR UL carrier |  | Conf 1, 4, 7 | G-FR1-A3-10 in [13] | G-FR1-A3-10 in [13] | G-FR1-A3-10 in [13] |  | N/A | G-FR1-A3-10 in [13] | N/A |
|  |  | Conf 2, 5, 8 | G-FR1-A3-10 in [13] | G-FR1-A3-10 in [13] | G-FR1-A3-10 in [13] |  | N/A | G-FR1-A3-10 in [13] | N/A |
|  |  | Conf 3, 6, 9 | G-FR1-A3-14 in [13] | G-FR1-A3-14 in [13] | G-FR1-A3-14 in [13] |  | N/A | G-FR1-A3-14 in [13] | N/A |
| PUCCH parameters For NR UL carrier |  | Conf 1, 4, 7 | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] |  | N/A | N/A | N/A |
|  |  | Conf 2, 5, 8 | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] |  | N/A | N/A | N/A |
|  |  | Conf 3, 6, 9 | Table 8.3.3.1.2-2 in [13] | Table 8.3.3.1.2-2 in [13] | Table 8.3.3.1.2-2 in [13] |  | N/A | N/A | N/A |
| PUSCH parameters for supplementary UL |  | Conf 1, 4, 7 | N/A | G-FR1-A3-10 in [13] | N/A |  | G-FR1-A3-10 in [13] | G-FR1-A3-10 in [13] | G-FR1-A3-10 in [13] |
|  |  | Conf 2, 5, 8 | N/A | G-FR1-A3-10 in [13] | N/A |  | G-FR1-A3-10 in [13] | G-FR1-A3-10 in [13] | G-FR1-A3-10 in [13] |
|  |  | Conf 3, 6, 9 | N/A | G-FR1-A3-14 in [13] | N/A |  | G-FR1-A3-14 in [13] | G-FR1-A3-14 in [13] | G-FR1-A3-14 in [13] |
| PUCCH parameters for supplementary UL |  | Conf 1, 4, 7 | N/A | N/A | N/A |  | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] |
|  |  | Conf 2, 5, 8 | N/A | N/A | N/A |  | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] |
|  |  | Conf 3, 6, 9 | N/A | N/A | N/A |  | Table 8.3.3.1.2-2 in [13] | Table 8.3.3.1.2-2 in [13] | Table 8.3.3.1.2-2 in [13] |
| PDSCH reference measurement channel as defined in A.3.1.1 |  | Conf 1, 4, 7 | SR.1.1 FDD |  |  |  | SR.1.1 FDD |  |  |
|  |  | Conf 2, 5, 8 | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |
|  |  | Conf 3, 6, 9 | SR 2.1 TDD |  |  |  | SR 2.1 TDD |  |  |
| RMSI CORESET reference measurement channel as defined in A.3.1.2 |  | Conf 1, 4, 7 | CR.1.1 FDD |  |  |  | CR.1.1 FDD |  |  |
|  |  | Conf 2, 5, 8 | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |
|  |  | Conf 3, 6, 9 | CR.2.1 TDD |  |  |  | CR.2.1 TDD |  |  |
| RMC CORESET reference measurement channel as defined in A.3.1.3 |  | Conf 1, 4, 7 | CCR.1.1 FDD |  |  |  | CCR.1.1 FDD |  |  |
|  |  | Conf 2, 5, 8 | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |  |  |
|  |  | Conf 3, 6, 9 | CCR.2.1 TDD |  |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern Note 1 |  | Conf 1, 2, 4, 5, 7, 8 | OP.1 Note 4 |  |  |  | OP.1 Note 4 |  |  |
|  |  | Conf 3, 6, 9 | OP.1 Note 5 |  |  |  | OP.1 Note 5 |  |  |
| SSB configuration |  | Conf 1, 2, 4, 5, 7,8 | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |
|  |  | Conf 3, 6, 9 | SSB.2 FR1 |  |  |  | SSB.2 FR1 |  |  |
| SMTC configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | SMTC.1 |  |  |  | SMTC.1 |  |  |
| CSI-RS for tracking |  | Conf 1 | TRS.1.1 FDD |  |  |  | TRS.1.1 FDD |  |  |
|  |  | Conf 2 | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  |
|  |  | Conf 3 | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |
|  |  | Conf 4 | TRS.1.1 FDD |  |  |  | TRS.1.1 FDD |  |  |
|  |  | Conf 5 | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  |
|  |  | Conf 6 | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |
|  |  | Conf 7 | TRS.1.1 FDD |  |  |  | TRS.1.1 FDD |  |  |
|  |  | Conf 8 | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  |
|  |  | Conf 9 | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |
| DL initial BWP configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | DLBWP.0.1 |  |  |  | DLBWP.0.1 |  |  |
| DL dedicated BWP configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | DLBWP.1.1 |  |  |  | DLBWP.1.1 |  |  |
| UL dedicated BWP configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | ULBWP.1.1 |  |  |  | ULBWP.1.1 |  |  |
| EPRE ratio of PSS to SSS | dB | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 0 |  |  |  | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |  |  |  |
| Note 2 | dBm / 15kHz | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | -102 |  |  |  | -102 |  |  |
|  | dBm/ SCS | Conf 1, 2, 4, 5, 7,8 | -102 |  |  |  | -102 |  |  |
|  |  | Conf 3, 6, 9 | -99 |  |  |  | -99 |  |  |
|  | dB | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 16 | 16 |  | 16 | 16 | 16 | 16 |
| Note 3 | dB | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 16 | 16 |  | 16 | 16 | 16 | 16 |
| SS-RSRP Note 3 | dBm/ SCS | Conf 1, 2, 4, 5, 7,8 | -86 | -86 |  | -86 | -86 | -86 | -86 |
|  |  | Conf 3, 6, 9 | -83 | -83 |  | -83 | -83 | -83 | -83 |
| Io Note 3 | dBm/ 9.36 MHz | Conf 1, 2, 4, 5, 7,8 | -57.9 | -57.9 |  | -57.9 | -57.9 | -57.9 | -57.9 |
|  | dBm/ 38.16MHz | Conf 3, 6, 9 | -51.8 | -51.8 |  | -51.8 | -51.8 | -51.8 | -51.8 |
| Propagation Condition |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | AWGN |  |  |  | AWGN |  |  |
| Antenna configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 1 x 2 |  |  |  | 1 x 2 |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. NOTE 3: , Io, and SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 6: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |  |  |  |  |  |  |

##### A.6.5.4.1.2	Test Requirements

In test 1 the UE shall be ready to start transmission on the supplementary uplink carrier on SCell within 20ms from the start of T2.

In test 1 the UE shall stop the transmission on the supplementary uplink carrier on SCell within 20ms from the start of T3.

In test 2 the UE shall be ready to start transmission on the NR uplink carrier on SCell within 20ms from the start of T2.

In test 2 the UE shall stop the transmission on the NR uplink carrier on SCell within 20ms from the start of T3.

All of the above test requirements shall be fulfilled in order for the observed UE UL carrier configuration delay and UE UL carrier release delay to be counted as correct. The rate of correct observed UE UL carrier configuration delay and UE UL carrier release delay during repeated tests shall be at least 90%.

#### A.6.5.4.2	Void

### A.6.5.5	Beam Failure Detection and Link recovery procedures

#### A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode

##### A.6.5.5.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving cell and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR1 serving cell requirements in clause 8.5.

The test parameters are given in Tables A.6.5.5.1.1-1, A.6.5.5.1.1-2, A.6.5.5.1.1-3 and A.6.5.5.1.1-4 below. There is one cell, cell 1 which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.6.5.5.1.1-1 shows the variation of the downlink SNR of the SSB in set q0 in the active cell to emulate SSB based beam failure. Figure A.6.5.5.1.1-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5  ms. In the test, DRX configuration is not enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms) in test 1.

Table A.6.5.5.1.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 3 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.5.1.1-2: General test parameters for FR1 PCell for SSB-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  |  |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Test 1 |  |
| Active PSCell |  |  |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  |  |  | 1 |  |
| Duplex mode |  |  |  | Config 1 |  | FDD |  |
|  |  |  |  | Config 2, 3 |  | TDD |  |
| BWchannel |  |  |  | Config 1 | MHz | 10: NRB,c = 52 |  |
|  |  |  |  | Config 2 |  | 10: NRB,c = 52 |  |
|  |  |  |  | Config 3 |  | 40: NRB,c = 106 |  |
| DL initial BWP configuration |  |  |  | Config 1, 2, 3 |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  |  |  | Config 1, 2, 3 |  | DLBWP.1.1 |  |
| UL initial BWP configuration |  |  |  | Config 1, 2, 3 |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  |  |  | Config 1, 2, 3 |  | ULBWP.1.1 |  |
| TDD Configuration |  |  |  | Config 1 |  | Not Applicable |  |
|  |  |  |  | Config 2 |  | TDDConf.1.1 |  |
|  |  |  |  | Config 3 |  | TDDConf.2.1 |  |
| RMSI CORESET Reference Channel |  |  |  | Config 1 |  | CR.1.1 FDD |  |
|  |  |  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  |  |  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  |  |  | Config 1 |  | CCR.1.1 FDD |  |
|  |  |  |  | Config 2 |  | CCR.1.1 TDD |  |
|  |  |  |  | Config 3 |  | CCR.2.1 TDD |  |
| SSB Configuration |  |  |  | Config 1 |  | SSB.3 FR1 |  |
|  |  |  |  | Config 2 |  | SSB.3 FR1 |  |
|  |  |  |  | Config 3 |  | SSB.4 FR1 |  |
| SMTC Configuration |  |  |  | Config 1, 2 |  | SMTC.1 |  |
|  |  |  |  | Config 3 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  |  |  | Config 1, 2 |  | 15 KHz |  |
|  |  |  |  | Config 3 |  | 30 KHz |  |
| PRACH Configuration |  |  |  | Config 1, 2 |  | Table A.3.8.2.2-1 |  |
|  |  |  |  | Config 3 |  | Table A.3.8.2.2-1 |  |
| SSB Index assigned as BFD RS (q0) |  |  |  |  |  | 0 |  |
| SSB Index assigned as CBD RS (q1) |  |  |  |  |  | 1 |  |
| OCNG parameters |  |  |  |  |  | OP.1 |  |
| CP length |  |  |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  |  | 2x2 Low |  |
| Beam failure detection transmission parameters |  | DCI format |  |  |  | 1-0 |  |
|  |  | Number of Control OFDM symbols |  |  |  | 2 |  |
|  |  | Aggregation level |  |  | CCE | 8 |  |
|  |  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB | 0 |  |
|  |  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB | 0 |  |
|  |  | DMRS precoder granularity |  |  |  | REG bundle size |  |
|  |  | REG bundle size |  |  |  | 6 |  |
| DRX |  |  |  |  |  | OFF |  |
| Gap pattern ID |  |  |  |  |  | gp0 |  |
| gapOffset |  |  |  |  |  | 0 |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  |  | absent | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp-ThresholdSSB | Config 1, 2 |  |  |  | dBm/SCS kHz | -98 | Threshold used for Qin_LR_SSB |
|  | Config 3 |  |  |  |  | -95 |  |
| powerControlOffsetSS |  |  |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  |  |  | n1 | see clause 5.17 of TS 38.321 [7] |
| beamFailureDetectionTimer |  |  |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [7] |
| CSI-RS configuration for CSI reporting |  |  | Config 1 |  |  | CSI-RS.1.1 FDD |  |
|  |  |  | Config 2 |  |  | CSI-RS.1.1 TDD |  |
|  |  |  | Config 3 |  |  | CSI-RS.2.1 TDD |  |
| CSI-RS for tracking |  |  | Config 1 |  |  | TRS.1.1 FDD |  |
|  |  |  | Config 2 |  |  | TRS.1.1 TDD |  |
|  |  |  | Config 3 |  |  | TRS.1.2 TDD |  |
| SSB Index assigned as RLM RS |  |  |  |  |  | 0, 1 |  |
| T310 Timer |  |  |  |  | ms | 1000 |  |
| N310 |  |  |  |  |  | 2 |  |
| T1 |  |  |  |  | s | 0.2 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  |  | s | 0.37 |  |
| T3 |  |  |  |  | s | 0.24 |  |
| T4 |  |  |  |  | s | 0 |  |
| T5 |  |  |  |  | s | 0.17 |  |
| D1 |  |  |  |  | s | 0.13 |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |  |

Table A.6.5.5.1.1-3: Cell specific test parameters for FR1 PCell for SSB-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 | Config 1 | dB | 5 | -3 | -12 | -12 | -12 |
|  | Config 2 |  | 5 | -3 | -12 | -12 | -12 |
|  | Config 3 |  | 5 | -3 | -12 | -12 | -12 |
| SNR_SSB of set q1 | Config 1 | dB | -10 | -10 | 10 | 10 | 10 |
|  | Config 2 |  | -10 | -10 | 10 | 10 | 10 |
|  | Config 3 |  | -10 | -10 | 10 | 10 | 10 |
| SSB_RP of set q1 | Config 1 | dBm/SCS kHz | -108 | -108 | -88 | -88 | -88 |
|  | Config 2 |  | -108 | -108 | -88 | -88 | -88 |
|  | Config 3 |  | -105 | -105 | -85 | -85 | -85 |
|  | Config 1 | dBm/15 KHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.4.5.5.1.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. |  |  |  |  |  |  |  |

Table A.6.5.5.1.1-4: Void

Figure A.6.5.5.1.1-1: SNR and L1-RSRP variation SSB for SSB-based beam failure detection and link recovery testing in non-DRX mode

A.6.5.5.1.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 120+10 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode

##### A.6.5.5.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving cell and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR1 serving cell requirements in clause 8.5.

The test parameters are given in Tables A.6.5.5.2.1-1, A.6.5.5.2.1-2, A.6.5.5.2.1-3, A.6.5.5.2.1-4 and A.6.5.5.2.1-5 below. There is one cell, cell 1 which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.6.5.5.2.1-1 shows the variation of the downlink SNR of the SSB in set q0 in the active cell to emulate SSB based beam failure. Figure A.6.5.5.2.1-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5  ms. In the test, DRX configuration is enabled in PCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.6.5.5.2.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 3 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.5.2.1-2: General test parameters for FR1 PCell for SSB-based beam failure detection and link recovery testing in DRX mode

| Parameter |  |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 |  |
| Active PSCell |  |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  |  | 1 |  |
| Duplex mode |  |  | Config 1 |  | FDD |  |
|  |  |  | Config 2, 3 |  | TDD |  |
| BWchannel |  |  | Config 1 | MHz | 10: NRB,c = 52 |  |
|  |  |  | Config 2 |  | 10: NRB,c = 52 |  |
|  |  |  | Config 3 |  | 40: NRB,c = 106 |  |
| DL initial BWP configuration |  |  | Config 1, 2, 3 |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  |  | Config 1, 2, 3 |  | DLBWP.1.1 |  |
| UL initial BWP configuration |  |  | Config 1, 2, 3 |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  |  | Config 1, 2, 3 |  | ULBWP.1.1 |  |
| TDD Configuration |  |  | Config 1 |  | Not Applicable |  |
|  |  |  | Config 2 |  | TDDConf.1.1 |  |
|  |  |  | Config 3 |  | TDDConf.2.1 |  |
| RMSI CORESET Reference Channel |  |  | Config 1 |  | CR.1.1 FDD |  |
|  |  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  |  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 |  | CCR.1.1 FDD |  |
|  |  |  | Config 2 |  | CCR.1.1 TDD |  |
|  |  |  | Config 3 |  | CCR.2.1 TDD |  |
| SSB Configuration |  |  | Config 1 |  | SSB.3 FR1 |  |
|  |  |  | Config 2 |  | SSB.3 FR1 |  |
|  |  |  | Config 3 |  | SSB.4 FR1 |  |
| SMTC Configuration |  |  | Config 1, 2 |  | SMTC.1 |  |
|  |  |  | Config 3 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1, 2 |  | 15 KHz |  |
|  |  |  | Config 3 |  | 30 KHz |  |
| PRACH Configuration |  |  | Config 1, 2 |  | Table A.3.8.2.2-1 |  |
|  |  |  | Config 3 |  | Table A.3.8.2.2-1 |  |
| SSB Index assigned as BFD RS (q0) |  |  |  |  | 0 |  |
| SSB Index assigned as CBD RS (q1) |  |  |  |  | 1 |  |
| OCNG parameters |  |  |  |  | OP.1 |  |
| CP length |  |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 2x2 Low |  |
| Beam failure detection transmission parameters |  | DCI format |  |  | 1-0 |  |
|  |  | Number of Control OFDM symbols |  |  | 2 |  |
|  |  | Aggregation level |  | CCE | 8 |  |
|  |  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 0 |  |
|  |  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 0 |  |
|  |  | DMRS precoder granularity |  |  | REG bundle size |  |
|  |  | REG bundle size |  |  | 6 |  |
| DRX |  |  |  |  | DRX.7 | A.3.3.7 |
| Gap pattern ID |  |  |  |  | N.A. |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  | Absent | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp-ThresholdSSB | Config 1, 2 |  |  | dBm/SCS kHz | -98 | Threshold used for Qin_LR_SSB |
|  | Config 3 |  |  |  | -95 |  |
| powerControlOffsetSS |  |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  |  | n1 | see clause 5.17 of TS 38.321 [7] |
| beamFailureDetectionTimer |  |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [7] |
| CSI-RS configuration for CSI reporting |  | Config 1 |  |  | CSI-RS.1.1 FDD |  |
|  |  | Config 2 |  |  | CSI-RS.1.1 TDD |  |
|  |  | Config 3 |  |  | CSI-RS.2.1 TDD |  |
| CSI-RS for tracking |  | Config 1 |  |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  |  | TRS.1.2 TDD |  |
| SSB Index assigned as RLM RS |  |  |  |  | 0, 1 |  |
| T310 Timer |  |  |  | ms | 1000 |  |
| N310 |  |  |  |  | 2 |  |
| T1 |  |  |  | s | 1 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  | s | 5.17 |  |
| T3 |  |  |  | s | 3.24 |  |
| T4 |  |  |  | s | 0 |  |
| T5 |  |  |  | s | 1.97 |  |
| D1 |  |  |  | s | 1.93 |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |

Table A.6.5.5.2.1-3: Cell specific test parameters for FR1 PCell for SSB-based beam failure detection and link recovery testing in DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 | Config 1 | dB | 5 | -3 | -12 | -12 | -12 |
|  | Config 2 |  | 5 | -3 | -12 | -12 | -12 |
|  | Config 3 |  | 5 | -3 | -12 | -12 | -12 |
| SNR_SSB of set q1 | Config 1 | dB | -10 | -10 | 10 | 10 | 10 |
|  | Config 2 |  | -10 | -10 | 10 | 10 | 10 |
|  | Config 3 |  | -10 | -10 | 10 | 10 | 10 |
| SSB_RP of set q1 | Config 1 | dBm/SCS kHz | -108 | -108 | -88 | -88 | -88 |
|  | Config 2 |  | -108 | -108 | -88 | -88 | -88 |
|  | Config 3 |  | -105 | -105 | -85 | -85 | -85 |
|  | Config 1 | dBm/15 KHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.4.5.5.1.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. |  |  |  |  |  |  |  |

Table A.6.5.5.2.1-4: Void

Table A.6.5.5.2.1-5: Void

Figure A.6.5.5.2.1-1: SNR and L1-RSRP variation for SSB-based beam failure detection and link recovery testing in non-DRX mode

##### A.6.5.5.2.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 1920+10 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode

##### A.6.5.5.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects CSI-RS-based beam failure in the set q0 configured for a serving cell and that the UE performs correct CSI-RS-based link recovery based on beam candicate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the CSI-RS based beam failure detection and link recovery for an FR1 serving cell requirements in clause 8.5.

The test parameters are given in Tables A.6.5.5.3.1-1, A.6.5.5.3.1-2, and below. There is one cell, cell 1 which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.6.5.5.3.1-1 shows the variation of the downlink SNR of the CSI-RS in set q0 in the active cell to emulate CSI-RS based beam failure. Figure A.6.5.5.3.1-1 additionally shows the variation of the downlink L1-RSRP of the CSI-RS in set q1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is not enabled.

Table A.6.5.5.3.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 3 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.5.3.1-2: General test parameters for FR1 PCell for CSI-RS-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 |  |
| Active PCell |  |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  |  | 1 |  |
| Duplex mode | Config 1 |  |  |  | FDD |  |
|  | Config 2, 3 |  |  |  | TDD |  |
| TDD Configuration | Config 1 |  |  |  | Not Applicable |  |
|  | Config 2 |  |  |  | TDDConf.1.1 |  |
|  | Config 3 |  |  |  | TDDConf.2.1 |  |
| RMSI CORESET Reference Channel | Config 1 |  |  |  | CR.1.1 FDD | A.3.1.2 |
|  | Config 2 |  |  |  | CR.1.1 TDD |  |
|  | Config 3 |  |  |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel | Config 1 |  |  |  | CCR.1.1 FDD | A.3.1.3 |
|  | Config 2 |  |  |  | CCR.1.1 TDD |  |
|  | Config 3 |  |  |  | CCR.2.1 TDD |  |
| SSB Configuration | Config 1 |  |  |  | SSB.3 FR1 | A.3.10 |
|  | Config 2 |  |  |  | SSB.3 FR1 |  |
|  | Config 3 |  |  |  | SSB.4 FR1 |  |
| SSB Configuration | Config 1 |  |  |  | SSB. 3 FR1 | A.3.10 |
|  | Config 2 |  |  |  | SSB. 3 FR1 |  |
|  | Config 3 |  |  |  | SSB. 4 FR1 |  |
| SMTC Configuration | Config 1, 2 |  |  |  | SMTC.1 | A.3.11 |
|  | Config 3 |  |  |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1, 2 |  |  |  | 15 KHz |  |
|  | Config 3 |  |  |  | 30 KHz |  |
| PRACH Configuration | Config 1, 2 |  |  |  | FR1 PRACH configuration 4 | A.3.8.2 |
|  | Config 3 |  |  |  | FR1 PRACH configuration 4 | A.3.8.2 |
| csi-RS-Index assigned as beam failure detection RS in set q0 |  |  |  |  | 0 |  |
| OCNG parameters |  |  |  |  | OP.1 | A.3.2.1 |
| CP length |  |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 2x2 Low |  |
| Beam failure detection transmission parameters | DCI format |  |  |  | 1-0 |  |
|  | Number of Control OFDM symbols |  |  |  | 2 |  |
|  | Aggregation level |  |  | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  |  | REG bundle size |  |
|  | REG bundle size |  |  |  | 6 |  |
| DRX |  |  |  |  | OFF |  |
| Gap pattern ID |  |  |  |  | N.A. |  |
| csi-RS-Index assigned as candidate beam detection RS in set q1 |  |  |  |  | 1 | N |
| rlmInSyncOutOfSyncThreshold |  |  |  |  | absent | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp-ThresholdSSB |  | Config 1, 2 |  | dBm/SCS kHz | -98 | Threshold used for Qin_LR_SSB |
|  |  | Config 3 |  |  | -95 |  |
| powerControlOffsetSS |  |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  |  | n1 | see clause 5.17 of TS 38.321 [7] |
| beamFailureDetectionTimer |  |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [7] |
| CSI-RS configuration for q0 and q1 |  |  | Config 1 |  | CSI-RS.1.2 FDD | A.3.14 |
|  |  |  | Config 2 |  | CSI-RS.1.2 TDD |  |
|  |  |  | Config 3 |  | CSI-RS.2.2 TDD |  |
| CSI-RS configuration for CSI reporting |  |  | Config 1 |  | CSI-RS.1.1 FDD | A.3.14 |
|  |  |  | Config 2 |  | CSI-RS.1.1 TDD |  |
|  |  |  | Config 3 |  | CSI-RS.2.1 TDD |  |
| TRS configuration |  |  | Config 1 |  | TRS.1.1 FDD |  |
|  |  |  | Config 2 |  | TRS.1.1 TDD |  |
|  |  |  | Config 3 |  | TRS.1.2 TDD |  |
| CSI-RS-Index assigned as RLM RS |  |  | Config 1 |  | CSI-RS.1.2 FDD | A.3.14 |
|  |  |  | Config 2 |  | CSI-RS.1.2 TDD |  |
|  |  |  | Config 3 |  | CSI-RS.2.2 TDD |  |
| T310 Timer |  |  |  | ms | 1000 |  |
| N310 |  |  |  |  | 2 |  |
| T1 |  |  |  | s | 0.2 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  | s | 0.18 |  |
| T3 |  |  |  | s | 0.14 |  |
| T4 |  |  |  | s | 0 |  |
| T5 |  |  |  | s | 0.08 |  |
| D1 |  |  |  | s | 0.04 |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |

Table A.6.5.5.3.1-3: Cell specific test parameters for FR1 PCell for CSI-RS-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_CSI-RS of set q0 | Config 1 | dB | 5 | -3 | -12 | -12 | -12 |
|  | Config 2 |  | 5 | -3 | -12 | -12 | -12 |
|  | Config 3 |  | 5 | -3 | -12 | -12 | -12 |
| SNR_CSI-RS of set q1 | Config 1 | dB | -10 | -10 | 10 | 10 | 10 |
|  | Config 2 |  | -10 | -10 | 10 | 10 | 10 |
|  | Config 3 |  | -10 | -10 | 10 | 10 | 10 |
| CSI-RS_RP of set q1 | Config 1 | dBm/SCS kHz | -108 | -108 | -88 | -88 | -88 |
|  | Config 2 |  | -108 | -108 | -88 | -88 | -88 |
|  | Config 3 |  | -105 | -105 | -85 | -85 | -85 |
|  | Config 1 | dBm/15 KHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.4.5.5.1.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. |  |  |  |  |  |  |  |

Table A.6.5.5.3.1-4: Void

Table A.6.5.5.3.1-5: Void

Figure A.6.5.5.3.1-1: SNR and L1-RSRP variation for CSI-RS-based beam failure detection and link recovery testing in non-DRX mode

##### A.6.5.5.3.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the shall detect beam failure and initiat link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 30+10 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode

##### A.6.5.5.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects CSI-RS-based beam failure in the set q0 configured for a serving cell and that the UE performs correct CSI-RS-based link recovery based on beam candicate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the CSI-RS based beam failure detection and link recovery for an FR1 serving cell requirements in clause 8.5.

The test parameters are given in Tables A.6.5.5.4.1-1, A.6.5.5.4.1-2, A.6.5.5.4.1-3, and A.6.5.5.4.1-4 below. There is one cell, cell 1 which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.6.5.5.4.1-1 shows the variation of the downlink SNR of the CSI-RS in set q0 in the active cell to emulate CSI-RS based beam failure. Figure A.6.5.5.4.1-1 additionally shows the variation of the downlink L1-RSRP of the CSI-RS in set q1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5  ms. In the test, DRX configuration is enabled in PCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.6.5.5.4.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 3 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.5.4.1-2: General test parameters for FR1 PCell for CSI-RS-based beam failure detection and link recovery testing in DRX mode

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |
| Active PCell |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  | 1 |  |
| Duplex mode | Config 1 |  |  | FDD |  |
|  | Config 2, 3 |  |  | TDD |  |
| TDD Configuration | Config 1 |  |  | Not Applicable |  |
|  | Config 2 |  |  | TDDConf.1.1 |  |
|  | Config 3 |  |  | TDDConf..21 |  |
| RMSI CORESET Reference Channel | Config 1 |  |  | CR.1.1 FDD | A.3.1.2 |
|  | Config 2 |  |  | CR.1.1 TDD |  |
|  | Config 3 |  |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel | Config 1 |  |  | CCR.1.1 FDD | A.3.1.3 |
|  | Config 2 |  |  | CCR.1.1 TDD |  |
|  | Config 3 |  |  | CCR.2.1 TDD |  |
| SSB Configuration | Config 1 |  |  | SSB. 3 FR1 | A.3.10 |
|  | Config 2 |  |  | SSB. 3 FR1 |  |
|  | Config 3 |  |  | SSB. 4 FR1 |  |
| SMTC Configuration | Config 1, 2 |  |  | SMTC.1 | A.3.11 |
|  | Config 3 |  |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1, 2 |  |  | 15 KHz |  |
|  | Config 3 |  |  | 30 KHz |  |
| PRACH Configuration | Config 1, 2 |  |  | FR1 PRACH configuration 4 | A.3.8.2 |
|  | Config 3 |  |  | FR1 PRACH configuration 4 | A.3.8.2 |
| csi-RS-Index assigned as beam failure detection RS in set q0 |  |  |  | 0 |  |
| OCNG parameters |  |  |  | OP.1 | A.3.2.1 |
| CP length |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |  |
| Beam failure detection transmission parameters | DCI format |  |  | 1-0 |  |
|  | Number of Control OFDM symbols |  |  | 2 |  |
|  | Aggregation level |  | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  | REG bundle size |  |
|  | REG bundle size |  |  | 6 |  |
| DRX |  |  |  | DRX.7 | A.3.3.7 |
| Gap pattern ID |  |  |  | N.A. |  |
| csi-RS-Index assigned as candidate beam detection RS in set q1 |  |  |  | 1 |  |
| rlmInSyncOutOfSyncThreshold |  |  |  | absent | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp-ThresholdSSB |  | Config 1, 2 | dBm/SCS kHz | -98 | Threshold used for Qin_LR_SSB |
|  |  | Config 3 |  | -95 |  |
| powerControlOffsetSS |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  | n1 | see clause 5.17 of TS 38.321 [7] |
| beamFailureDetectionTimer |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [7] |
| CSI-RS configuration for q0 and q1 | Config 1 |  |  | CSI-RS.1.2 FDD | A.3.14 .1 |
|  | Config 2 |  |  | CSI-RS.1.2 TDD |  |
|  | Config 3 |  |  | CSI-RS.2.2 TDD |  |
| CSI-RS configuration for CSI reporting | Config 1 |  |  | CSI-RS.1.1 FDD | A.3.14.1 |
|  | Config 2 |  |  | CSI-RS.1.1 TDD |  |
|  | Config 3 |  |  | CSI-RS.2.1 TDD |  |
| TRS configuration | Config 1 |  |  | TRS.1.1 FDD |  |
|  | Config 2 |  |  | TRS.1.1 TDD |  |
|  | Config 3 |  |  | TRS.1.2 TDD |  |
| CSI-RS-Index assigned as RLM RS | Config 1 |  |  | CSI-RS.1.2 FDD |  |
|  | Config 2 |  |  | CSI-RS.1.2 TDD |  |
|  | Config 3 |  |  | CSI-RS.2.2 TDD |  |
| T310 Timer |  |  | ms | 1000 |  |
| N310 |  |  |  | 2 |  |
| T1 |  |  | s | 1 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  |  | s | 8.37 |  |
| T3 |  |  | s | 6.44 |  |
| T4 |  |  | s | 0 |  |
| T5 |  |  | s | 1.97 |  |
| D1 |  |  | s | 1.93 |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

Table A.6.5.5.4.1-3: Cell specific test parameters for FR1 PCell for CSI-RS-based beam failure detection and link recovery testing in DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_CSI-RS of set q0 | Config 1 | dB | 5 | -3 | -12 | -12 | -12 |
|  | Config 2 |  | 5 | -3 | -12 | -12 | -12 |
|  | Config 3 |  | 5 | -3 | -12 | -12 | -12 |
| SNR_CSI-RS of set q1 | Config 1 | dB | -10 | -10 | 10 | 10 | 10 |
|  | Config 2 |  | -10 | -10 | 10 | 10 | 10 |
|  | Config 3 |  | -10 | -10 | 10 | 10 | 10 |
| CSI-RS_RP of set q1 | Config 1 | dB/SCS kHz | -110 | -110 | -88 | -88 | -88 |
|  | Config 2 |  | -110 | -110 | -88 | -88 | -88 |
|  | Config 3 |  | -107 | -107 | -85 | -85 | -85 |
|  | Config 1 | dBm/15 KHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.4.5.5.1.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. |  |  |  |  |  |  |  |

Table A.6.5.5.4.1-4: Void

Table A.6.5.5.4.1-5: Void

Table A.6.5.5.4.1-6: Void

Figure A.6.5.5.4.1-1: SNR and L1-RSRP variation for CSI-RS-based beam failure detection and link recovery testing in DRX mode

##### A.6.5.5.4.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the shall detect beam failure and initiat link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 1920+10 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.5.5	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode

##### A.6.5.5.5.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects CSI-RS-based beam failure in the set q0 configured for a serving cell and that the UE performs correct SSB-based link recovery based on beam candicate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP without schedulingRequestID-BFR-SCell-r16 configuration, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the beam failure detection and link recovery for an FR1 serving cell requirements in clause 8.5.

The test parameters are given in Tables A.6.5.5.5.1-1, A.6.5.5.5.1-2, and below. There are two cells, cell 1 is the PCell and cell 2 is the SCell, in the test. UE is not provided by schedulingRequestID-BFR-SCell-r16, i.e., no configuration for PUCCH transmission resources, and UE shall perform the random access procedure to recover the beam failure. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.6.5.5.5.1-1 shows the SNR of the CSI-RS in set q0 in the active SCell to emulate beam failure. Figure A.6.5.5.5.1-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery.  Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is not enabled.

Table A.6.5.5.5.1-1: Supported test configurations for FR1 PCell and SCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 3 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.5.5.1-2: General test parameters for FR1 SCell for beam failure detection and link recovery testing in non-DRX mode

| Parameter |  |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 |  |
| Active PCell |  |  |  |  | Cell 1 |  |
| RF Channel Number for PCell |  |  |  |  | 1 |  |
| Active SCell |  |  |  |  | Cell 2 |  |
| RF Channel Number for SCell |  |  |  |  | 2 |  |
| Duplex mode | Config 1 |  |  |  | FDD |  |
|  | Config 2, 3 |  |  |  | TDD |  |
| BW channel | Config 1 |  |  |  | 10: NRB,c = 52 |  |
|  | Config 2 |  |  | MHz | 10: NRB,c = 52 |  |
|  | Config 3 |  |  |  | 40: NRB,c = 106 |  |
| TDD Configuration | Config 1 |  |  |  | Not Applicable |  |
|  | Config 2 |  |  |  | TDDConf.1.1 |  |
|  | Config 3 |  |  |  | TDDConf.2.1 |  |
| CORESET Reference Channel | Config 1 |  |  |  | CR.1.1 FDD | A.3.1.2 |
|  | Config 2 |  |  |  | CR.1.1 TDD |  |
|  | Config 3 |  |  |  | CR.2.1 TDD |  |
| SSB Configuration | Config 1 |  |  |  | SSB.1 FR1 | A.3.10 |
|  | Config 2 |  |  |  | SSB.1 FR1 |  |
|  | Config 3 |  |  |  | SSB.2 FR1 |  |
| SMTC Configuration | Config 1, 2 |  |  |  | SMTC.1 | A.3.11 |
|  | Config 3 |  |  |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1, 2 |  |  | kHz | 15 |  |
|  | Config 3 |  |  |  | 30 |  |
| PRACH Configuration | Config 1, 2, 4, 5 |  |  |  | Table A.3.8.2.2-1 |  |
|  | Config 3, 6 |  |  |  | Table A.3.8.2.2-1 |  |
| csi-RS-Index assigned as beam failure detection RS in set q0 in activated SCell |  |  |  |  | 0 |  |
| OCNG parameters |  |  |  |  | OP.1 | A.3.2.1 |
| CP length |  |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 2x2 Low |  |
| Beam failure detection transmission parameters | DCI format |  |  |  | 1-0 |  |
|  | Number of Control OFDM symbols |  |  |  | 2 |  |
|  | Aggregation level |  |  | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  |  | REG bundle size |  |
|  | REG bundle size |  |  |  | 6 |  |
| DRX |  |  |  |  | OFF |  |
| Gap pattern ID |  |  |  |  | N.A. |  |
| schedulingRequestID-BFR-SCell-r16 |  |  |  |  | absent | When the field is absent, the random access procedure will be triggered for SCell BFR |
| SSB Index assigned as CBD RS (q1) in activated SCell |  |  |  |  | 0 |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  | absent | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp-ThresholdBFR |  | Config 1, 2 |  | dBm/SCS kHz | -98 | Threshold used for Qin_LR_SSB |
|  |  | Config 3 |  |  | -95 |  |
| powerControlOffsetSS |  |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  |  | n1 | see clause 5.17 of TS 38.321 [7] |
| beamFailureDetectionTimer |  |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [7] |
| CSI-RS configuration for q0 in activated SCell |  |  | Config 1 |  | CSI-RS.1.2 FDD | A.3.14 |
|  |  |  | Config 2 |  | CSI-RS.1.2 TDD |  |
|  |  |  | Config 3 |  | CSI-RS.2.2 TDD |  |
| CSI-RS configuration for CSI reporting |  |  | Config 1 |  | CSI-RS.1.1 FDD | A.3.14 |
|  |  |  | Config 2 |  | CSI-RS.1.1 TDD |  |
|  |  |  | Config 3 |  | CSI-RS.2.1 TDD |  |
| TRS configuration |  |  | Config 1 |  | TRS.1.1 FDD |  |
|  |  |  | Config 2 |  | TRS.1.1 TDD |  |
|  |  |  | Config 3 |  | TRS.1.2 TDD |  |
| CSI-RS-Index assigned as RLM RS in PCell |  |  | Config 1 |  | CSI-RS.1.2 FDD | A.3.14 |
|  |  |  | Config 2 |  | CSI-RS.1.2 TDD |  |
|  |  |  | Config 3 |  | CSI-RS.2.2 TDD |  |
| T310 Timer |  |  |  | ms | 1000 |  |
| N310 |  |  |  |  | 2 |  |
| T1 |  |  |  | s | 0.2 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  | s | 0.18 |  |
| T3 |  |  |  | s | 0.14 |  |
| T4 |  |  |  | s | 0 |  |
| T5 |  |  |  | s | 0.17 |  |
| D1 |  |  |  | s | 0.13 |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |

Table A.6.5.5.5.1-3: Cell specific test parameters for FR1 SCell for CSI-RS-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  | Unit | Cell1 | Test 1 Cell2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 to T5 | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB |  | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |  |
| SNR_CSI-RS of set q0 | Config 1 | dB | 5 | 5 | -3 | -12 | -12 | -12 |
|  | Config 2 |  | 5 | 5 | -3 | -12 | -12 | -12 |
|  | Config 3 |  | 5 | 5 | -3 | -12 | -12 | -12 |
| SNR_SSB of set q1 | Config 1 | dB | -10 | -10 | -10 | 10 | 10 | 10 |
|  | Config 2 |  | -10 | -10 | -10 | 10 | 10 | 10 |
|  | Config 3 |  | -10 | -10 | -10 | 10 | 10 | 10 |
| SSB_RP of set q1 | Config 1 | dBm/ | -108 | -108 | -108 | -88 | -88 | -88 |
|  | Config 2 | SCS kHz | -108 | -108 | -108 | -88 | -88 | -88 |
|  | Config 3 |  | -105 | -105 | -105 | -85 | -85 | -85 |
|  | Config 1 | dBm/ | -98 | -98 |  |  |  |  |
|  | Config 2 | 15kHz | -98 | -98 |  |  |  |  |
|  | Config 3 |  | -98 | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.4.5.5.1.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. |  |  |  |  |  |  |  |  |

Figure A.6.5.5.5.1-1: SNR and L1-RSRP variation for beam failure detection and link recovery testing in for SCell non-DRX mode

##### A.6.5.5.5.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the shall detect beam failure and initiat link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 120+10 ms after the start of T5, the UE shall transmit preamble for UL-SCH resource application, followed by MAC-CE on the assigned uplink resources containing  a beam associated with the candidate beam set q1. The UE shall not transmit preamble earlier than time point B.

During T5, the System Simulator shall transmit a Random Access Response to UE after the System Simulator receives the preamble from UE. The UE shall transmit the msg.3 containing candidate beam set q1 for SCell BFR if UE receives the Random Access Response.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.5.6	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode

##### A.6.5.5.6.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects CSI-RS-based  beam failure in the set q0 configured for a serving cell and that the UE performs correct SSB-based link recovery based on beam candicate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP without schedulingRequestID-BFR-SCell-r16 configuration, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the beam failure detection and link recovery for an FR1 serving cell requirements in clause 8.5.

The test parameters are given in Tables A.6.5.5.6.1-1, A.6.5.5.6.1-2, A.6.5.5.6.1-3, and A.6.5.5.6.1-4 below. There are two cells, cell 1 is the PCell and cell 2 is the SCell, in the test. UE is not provided by schedulingRequestID-BFR-SCell-r16, i.e., no configuration for PUCCH transmission resources, and UE shall perform the random access procedure to recover the beam failure. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.6.5.5.6.1-1 shows the SNR of the CSI-RS in set q0 in the active SCell to emulate beam failure. Figure A.6.5.5.6.1-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is enabled in SCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.6.5.5.6.1-1: Supported test configurations for FR1 PCell and SCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 3 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.5.6.1-2: General test parameters for FR1 SCell for beam failure detection and link recovery testing in DRX mode

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |
| Active PCell |  |  |  | Cell 1 |  |
| RF Channel Number for PCell |  |  |  | 1 |  |
| Active SCell |  |  |  | Cell 2 |  |
| RF Channel Number for SCell |  |  |  | 2 |  |
| Duplex mode | Config 1 |  |  | FDD |  |
|  | Config 2, 3 |  |  | TDD |  |
| BW channel | Config 1 |  |  | 10: NRB,c = 52 |  |
|  | Config 2 |  | MHz | 10: NRB,c = 52 |  |
|  | Config 3 |  |  | 40: NRB,c = 106 |  |
| TDD Configuration | Config 1 |  |  | Not Applicable |  |
|  | Config 2 |  |  | TDDConf.1.1 |  |
|  | Config 3 |  |  | TDDConf..21 |  |
| CORESET Reference Channel | Config 1 |  |  | CR.1.1 FDD | A.3.1.2 |
|  | Config 2 |  |  | CR.1.1 TDD |  |
|  | Config 3 |  |  | CR.2.1 TDD |  |
| SSB Configuration | Config 1 |  |  | SSB.1 FR1 | A.3.10 |
|  | Config 2 |  |  | SSB.1 FR1 |  |
|  | Config 3 |  |  | SSB.2 FR1 |  |
| SMTC Configuration | Config 1, 2 |  |  | SMTC.1 | A.3.11 |
|  | Config 3 |  |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1, 2 |  |  | 15 KHz |  |
|  | Config 3 |  |  | 30 KHz |  |
| PRACH Configuration | Config 1, 2, 4, 5 |  |  | Table A.3.8.2.1-1 |  |
|  | Config 3, 6 |  |  | Table A.3.8.2.1-1 |  |
| csi-RS-Index assigned as beam failure detection RS in set q0 in activated SCell |  |  |  | 0 |  |
| OCNG parameters |  |  |  | OP.1 | A.3.2.1 |
| CP length |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |  |
| Beam failure detection transmission parameters | DCI format |  |  | 1-0 |  |
|  | Number of Control OFDM symbols |  |  | 2 |  |
|  | Aggregation level |  | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  | REG bundle size |  |
|  | REG bundle size |  |  | 6 |  |
| DRX |  |  |  | DRX.7 | A.3.3.7 |
| Gap pattern ID |  |  |  | N.A. |  |
| schedulingRequestID-BFR-SCell-r16 |  |  |  | absent | When the field is absent, the random access procedure will be triggered for SCell BFR |
| SSB Index assigned as CBD RS (q1) in activated SCell |  |  |  | 0 |  |
| rlmInSyncOutOfSyncThreshold |  |  |  | absent | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp-ThresholdBFR |  | Config 1, 2 | dBm/SCS kHz | -98 | Threshold used for Qin_LR_SSB |
|  |  | Config 3 |  | -95 |  |
| powerControlOffsetSS |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  | n1 | see clause 5.17 of TS 38.321 [7] |
| beamFailureDetectionTimer |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [7] |
| CSI-RS configuration for q0in activated SCell | Config 1 |  |  | CSI-RS.1.2 FDD | A.3.14.1 |
|  | Config 2 |  |  | CSI-RS.1.2 TDD |  |
|  | Config 3 |  |  | CSI-RS.2.2 TDD |  |
| CSI-RS configuration for CSI reporting | Config 1 |  |  | CSI-RS.1.1 FDD | A.3.14.1 |
|  | Config 2 |  |  | CSI-RS.1.1 TDD |  |
|  | Config 3 |  |  | CSI-RS.2.1 TDD |  |
| TRS configuration | Config 1 |  |  | TRS.1.1 FDD |  |
|  | Config 2 |  |  | TRS.1.1 TDD |  |
|  | Config 3 |  |  | TRS.1.2 TDD |  |
| CSI-RS-Index assigned as RLM RS in PCell | Config 1 |  |  | CSI-RS.1.2 FDD |  |
|  | Config 2 |  |  | CSI-RS.1.2 TDD |  |
|  | Config 3 |  |  | CSI-RS.2.2 TDD |  |
| T310 Timer |  |  | ms | 1000 |  |
| N310 |  |  |  | 2 |  |
| T1 |  |  | s | 1 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  |  | s | 8.37 |  |
| T3 |  |  | s | 6.44 |  |
| T4 |  |  | s | 0 |  |
| T5 |  |  | s | 1.97 |  |
| D1 |  |  | s | 1.93 |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

Table A.6.5.5.6.1-3: Cell specific test parameters for FR1 SCell for beam failure detection and link recovery testing in DRX mode

| Parameter |  | Unit | Cell 1 | Test 1 Cell2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 to T5 | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB |  | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |  |
| SNR_CSI-RS of set q0 | Config 1 | dB | 5 | 5 | -3 | -12 | -12 | -12 |
|  | Config 2 | dB | 5 | 5 | -3 | -12 | -12 | -12 |
|  | Config 3 | dB | 5 | 5 | -3 | -12 | -12 | -12 |
| SNR_SSB of set q1 | Config 1 | dB | -10 | -10 | -10 | 10 | 10 | 10 |
|  | Config 2 | dB | -10 | -10 | -10 | 10 | 10 | 10 |
|  | Config 3 | dB | -10 | -10 | -10 | 10 | 10 | 10 |
| SSB_RP of set q1 | Config 1 | dBm/ | -108 | -108 | -108 | -88 | -88 | -88 |
|  | Config 2 | SCS kHz | -108 | -108 | -108 | -88 | -88 | -88 |
|  | Config 3 |  | -105 | -105 | -105 | -85 | -85 | -85 |
|  | Config 1 | dBm/15 | -98 | -98 |  |  |  |  |
|  | Config 2 | kHz | -98 | -98 |  |  |  |  |
|  | Config 3 |  | -98 | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.4.5.5.1.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. |  |  |  |  |  |  |  |  |

Figure A.6.5.5.6.1-1: SNR and L1-RSRP variation for beam failure detection and link recovery testing for SCell in DRX mode

##### A.6.5.5.6.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the shall detect beam failure and initiat link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 120+10 ms after the start of T5, the UE shall transmit preamble for UL-SCH resource application, followed by MAC-CE on the assigned uplink resources containing  a beam associated with the candidate beam set q1. The UE shall not transmit preamble earlier than time point B.

During T5, the System Simulator shall transmit a Random Access Response to UE after the System Simulator receives the preamble from UE. The UE shall transmit the msg.3 containing candidate beam set q1 for SCell BFR if UE receives the Random Access Response.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.5.7	TRP Specific Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode

##### A.6.5.5.7.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects TRP specific CSI-RS-based beam failure in the sets q0,0  and q0,1  configured for a serving cell and that the UE performs correct CSI-RS-based link recovery based on beam candidate set q1,0 and q1,1. The purpose is to test the downlink monitoring for beam failure detection on TRP1 within the UEs active DL BWP, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the CSI-RS based TRP specific beam failure detection and link recovery for an FR1 serving cell requirements in clause 8.18 with schedulingRequestID-BFR-r17 configured.

The test parameters are given in Tables A.6.5.5.7.1-1, A.6.5.5.7.1-2, A.6.5.5.7.1-3, and A.6.5.5.7.1-4 below. There is one cell, cell 1 which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.6.5.5.7.1-1 shows the variation of the downlink SNR of the CSI-RS in set q0,0  and q0,1  for TRP1 and TRP2 respectively to emulate CSI-RS based beam failure on TRP1. Figure A.6.5.5.7.1-1 additionally shows the variation of the downlink L1-RSRP of the CSI-RS in set q1,0 and q1,1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5  ms. In the test, DRX configuration is enabled in PCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.6.5.5.7.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 3 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.5.7.1-2: General test parameters for FR1 PCell for CSI-RS-based TRP specific beam failure detection and link recovery testing in DRX mode

| Parameter |  |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test1 |  |
| Active PCell |  |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  |  | 1 |  |
| Duplex mode |  | Config 1 |  |  | FDD |  |
|  |  | Config 2, 3 |  |  | TDD |  |
| TDD Configuration |  | Config 1 |  |  | Not Applicable |  |
|  |  | Config 2 |  |  | TDDConf.1.1 |  |
|  |  | Config 3 |  |  | TDDConf.2.1 |  |
| RMSI CORESET Reference Channel |  | Config 1 |  |  | CR.1.1 FDD | A.3.1.2 |
|  |  | Config 2 |  |  | CR.1.1 TDD |  |
|  |  | Config 3 |  |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  |  | CCR.1.1 FDD | A.3.1.3 |
|  |  | Config 2 |  |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  |  | CCR.2.1 TDD |  |
| SSB Configuration |  | Config 1 |  |  | SSB.3 FR1 | A.3.10 |
|  |  | Config 2 |  |  | SSB.3 FR1 |  |
|  |  | Config 3 |  |  | SSB.4 FR1 |  |
| SMTC Configuration |  | Config 1, 2 |  |  | SMTC.1 | A.3.11 |
|  |  | Config 3 |  |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2 |  |  | 15 KHz |  |
|  |  | Config 3 |  |  | 30 KHz |  |
| PRACH Configuration |  | Config 1, 2,3 |  |  | FR1 PRACH configuration 4 | A.3.8.2 |
| OCNG parameters |  |  |  |  | OP.1 | A.3.2.1 |
| CP length |  |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 2x2 Low |  |
| Beam failure detection transmission parameters | DCI format |  |  |  | 1-0 |  |
|  | Number of Control OFDM symbols |  |  |  | 2 |  |
|  | Aggregation level |  |  | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  |  | REG bundle size |  |
|  | REG bundle size |  |  |  | 6 |  |
| DRX |  |  |  |  | DRX.7 | A.3.3.7 |
| Gap pattern ID |  |  |  |  | N.A. |  |
| schedulingRequestID-BFR- r17 |  |  |  |  | Configured |  |
| Periodicity of PUCCH for SR configuration for BFR on PCell |  |  | Config 1 | slot | 5 | 5ms |
|  |  |  | Config 2, 3 |  | 10 |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  | absent | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp-ThresholdSSB |  | Config 1, 2 |  | dBm/SCS kHz | -98 | Threshold used for Qin_LR_SSB |
|  |  | Config 3 |  |  | -95 |  |
| powerControlOffsetSS |  |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  |  | n1 | see clause 5.17 of TS 38.321 [7] |
| beamFailureDetectionTimer |  |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [7] |
| CSI-RS configuration for TRP0 |  | Config 1 |  |  | CSI-RS.1.2 FDD | A.3.14.1 |
|  |  | Config 2 |  |  | CSI-RS.1.2 TDD |  |
|  |  | Config 3 |  |  | CSI-RS.2.2 TDD |  |
| CSI-RS configuration for TRP1 |  | Config 1 |  |  | CSI-RS.1.7 FDD |  |
|  |  | Config 2 |  |  | CSI-RS.1.6 TDD |  |
|  |  | Config 3 |  |  | CSI-RS.2.7 TDD |  |
| CSI-RS Index assigned as BFD RS (q0,0) |  |  |  |  | 0 |  |
| CSI-RS Index assigned as CBD RS (q1,0) |  |  |  |  | 1 |  |
| CSI-RS Index assigned as BFD RS (q0,1) |  |  |  |  | 2 |  |
| CSI-RS Index assigned as CBD RS (q1,1) |  |  |  |  | 3 |  |
| CSI-RS index assigned as RLM RS |  |  |  |  | 0,1,2,3 |  |
| CSI-RS configuration for CSI reporting |  | Config 1 |  |  | CSI-RS.1.1 FDD | A.3.14.1 |
|  |  | Config 2 |  |  | CSI-RS.1.1 TDD |  |
|  |  | Config 3 |  |  | CSI-RS.2.1 TDD |  |
| TRS configuration |  | Config 1 |  |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  |  | TRS.1.2 TDD |  |
| T310 Timer |  |  |  | ms | 1000 |  |
| N310 |  |  |  |  | 2 |  |
| T1 |  |  |  | s | 1 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  | s | 8.37 |  |
| T3 |  |  |  | s | 6.44 |  |
| T4 |  |  |  | s | 0 |  |
| T5 |  |  |  | s | 1.97 |  |
| D1 |  |  |  | s | 1.93 |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |

Table A.6.5.5.7.1-3: Cell specific test parameters for FR1 PCell for CSI-RS-based beam failure detection and link recovery testing in DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_CSI-RS of set q0,0 | Config 1 | dB | 5 | -3 | -12 | -12 | -12 |
|  | Config 2 |  | 5 | -3 | -12 | -12 | -12 |
|  | Config 3 |  | 5 | -3 | -12 | -12 | -12 |
| SNR_CSI-RS of set q0,1 | Config 1 | dB | 5 | 5 | 5 | 5 | 5 |
|  | Config 2 |  | 5 | 5 | 5 | 5 | 5 |
|  | Config 3 |  | 5 | 5 | 5 | 5 | 5 |
| SNR_CSI-RS of set q1,0 | Config 1 | dB | 0.2 | 0.2 | 20.2 | 20.2 | 20.2 |
|  | Config 2 |  | 0.2 | 0.2 | 20.2 | 20.2 | 20.2 |
|  | Config 3 |  | 0.2 | 0.2 | 20.2 | 20.2 | 20.2 |
| CSI-RS_RP of set q1,0 | Config 1 | dB/SCS kHz | -110 | -110 | -88 | -88 | -88 |
|  | Config 2 |  | -110 | -110 | -88 | -88 | -88 |
|  | Config 3 |  | -107 | -107 | -85 | -85 | -85 |
|  | Config 1 | dBm/15 KHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.4.5.5.1.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. |  |  |  |  |  |  |  |

Figure A.6.5.5.7.1-1: SNR and L1-RSRP variation for CSI-RS-based beam failure detection and link recovery testing in DRX mode

##### A.6.5.5.7.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1,0.

No later than time point F occurring no later than D1 = 1920+10 ms after the start of T5, the UE shall transmit PUCCH with LRR, followed by BFR MAC CE containing a beam associated with the candidate beam set q1,0. The UE shall not transmit PUCCH with an LRR with the candidate beam set q1,0 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

### A.6.5.6	Active BWP switch

#### A.6.5.6.1	DCI-based and Timer-based Active BWP Switch

##### A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of SCell with non-DRX in SA

A.6.5.6.1.1.1	Test Purpose and Environment

The purpose of this test is to verify the DL BWP switch delay requirement defined in clause 8.6, and interruption requirement on other active serving cell defined in clause 8.2.2.2.5.

The supported test configurations for PCell are shown in Table A.6.5.6.1.1.1-1 below. Supported test configurations for NR SCell are shown in table A.6.5.6.1.1.1-1A below. Test configuration for NR PCell and test configuration for NR SCell are chosen independently. The test scenario comprises of one NR PCell (Cell 1) and one SCell (Cell 2) as given in Table A.6.5.6.1.1.1-2. NR Cell-specific parameters are specified in Table A.6.5.6.1.1.1-3 and Table A.6.5.6.1.1.1-4 below.

PDCCHs indicating new transmissions shall be sent continuously on SCell (Cell 2) to ensure that the UE would have ACK/NACK sending except for the time duration when BWP is switching on Cell 2 and the time duration of T2.

PDCCHs indicating new transmissions shall be sent continuously on PCell (Cell 1) to ensure that the UE will have ACK/NACK sending.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), and Cell 2 (SCell) on radio channel 2 (SCC).

-	UE is configured with 2 different UE-specific downlink bandwidth parts for SCell, BWP-1 and BWP-2, in Cell 2 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

-	UE is configured with 1 UE-specific downlink bandwidth parts the same as initial BWP for PCell, BWP-0 in Cell 1 before starting the test.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 in SCell.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-0 in PCell.

-	UE is configured with a bwp-InactivityTimer timer value for SCell.

All cells have constant signal levels throughout the test.

The test consists of 3 successive time periods, with durations of T1, T2, and T3, respectively.

During T1,

Time period T1 starts when a DCI format 1_1 command for SCell DL BWP switch, sent from the test equipment to the UE, is received at the UE side in SCell’s slot # denoted i. The UE shall switch its bandwidth part from BWP-1 to BWP-2.

The UE shall be able to receive PDSCH no later than the first DL slot that occurs after the beginning of SCell’s DL slot (i+TBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK for the SCell on PCell no later than the first UL slot that occurs after the beginning of slot (i+TBWPswitchDelay+k1). The UE shall be continuously scheduled on SCell’s BWP-2 no later than the first DL slot that occurs after the beginning of slot (i+TBWPswitchDelay).

The starting time of PCell (Cell 1) interruption due to BWP switch on SCell shall occur within the BWP switch delay.

During T2, the test equipment won’t transmit DCI format for PDSCH reception on SCell (Cell 2).

During T3,

The time period T3 starts from the slot #j, where j is the first  slot of the subframe immediately after bwp-InactivityTimer timer expires. The UE should switch its bandwidth part from BWP-2 back to the default bandwidth part – BWP-1.

The UE shall be able to receive PDSCH no later than the first DL slot that occurs after the beginning of SCell’s slot (j+TBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK for the SCell on PCell at latest on the first UL slot that occurs after the beginning of slot (j+TBWPswitchDelay+k1). The UE shall be continuously scheduled on SCell’s BWP-1 no later than the first DL slot that occurs after the beginning of slot (j+TBWPswitchDelay).

The starting time of PCell (Cell 1) interruption due to BWP switch of SCell shall occur within the BWP switch delay.

The test equipment verifies the DL BWP switch time in SCell by counting the slots from the time when the BWP switch command is received or bwp-InactivityTimer timer expires till an ACK/NACK is received.

The test equipment verifies that potential interruption to PCell is carried out in the correct time span by monitoring ACK/NACK sent in PCell during BWP switch of SCell, respectively.

Table A.6.5.6.1.1.1-1: DL BWP switch supported test configurations for NR PCell

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table A.6.5.6.1.1.1-1A: DL BWP switch supported test configurations for NR SCell

| ConfigSCell | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table A.6.5.6.1.1.1-2: General test parameters for DL BWP switch in SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1, 2 | Two NR radio channels are used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active SCell |  | Cell 2 | SCell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | For both PCell and SCell |
| bwp-InactivityTimer | ms | 200 |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC. |
| Cell2 timing offset to cell1 | s | 3 | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. |
| T1 | s | 0.2 |  |
| T2 | s | 0.2 |  |
| T3 | s | 0.2 |  |

Table A.6.5.6.1.1.1-3: NR Cell specific test parameters for NR PCell for DL BWP switch in SA

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| Frequency Range |  |  | FR1 |
| Duplex mode | Config 1 |  | FDD |
|  | Config 2,3 |  | TDD |
| TDD configuration | Config 1 |  | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.1.2 |
| BWchannel | Config 1,2 |  | Note 7 |
|  | Config 3 |  | Note 7 |
| BWoccupied | Config 1,2 | RB | 52 Note 5 |
|  | Config 3 |  | 106 Note 6 |
| Active BWP ID |  |  | 0 |
| Initial DL BWP Configuration |  |  | DLBWP.0.2Note4 |
| Initial UL BWP Configuration |  |  | ULBWP.0.2Note4 |
| Active DL BWP-0 Configuration |  |  | DLBWP.0.2Note4 |
| Active DL BWP-1 Configuration |  |  | N.A. |
| Active DL BWP-2 Configuration |  |  | N.A. |
| Active UL BWP-0 Configuration |  |  | ULBWP.0.2Note4 |
| Active UL BWP-1 Configuration |  |  | N.A. |
| Active UL BWP-2 Configuration |  |  | N.A. |
| PDSCH Reference | Config 1 |  | SR.1.1 FDD |
| measurement channel | Config 2 |  | SR.1.1 TDD |
|  | Config 3 |  | SR.2.1 TDD |
| RMSI CORESET | Config 1 |  | CR.1.1 FDD |
| parameters | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET | Config 1 |  | CCR.1.2 FDD |
| parameters | Config 2 |  | CCR.1.2 TDD |
|  | Config 3 |  | CCR.2.4 TDD |
| TRS Configuration | Config 1 |  | TRS.1.1 FDD |
|  | Config 2 |  | TRS.1.1 TDD |
|  | Config 3 |  | TRS.1.2 TDD |
| OCNG Patterns | Config 1,2 |  | OP.1 Note 5 |
|  | Config 3 |  | OP.1 Note 6 |
| SSB Configuration | Config 1,2 |  | SSB.1 FR1 |
|  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration |  |  | SMTC.1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |
| NocNote 2 | Config 1,2 | dBm/SCS | -104 |
|  | Config 3 |  | -101 |
| NocNote 2 |  | dBm/15KHz | -104 |
| SS-RSRP Note 3 | Config 1,2 | dBm/SCS | -87 |
|  | Config 3 |  | -84 |
| Ês/Iot |  | dB | 17 |
| Ês/Noc |  | dB | 17 |
| IoNote3 | Config 1,2 | dBm/ 9.36MHz | -58.96 |
|  | Config 3 | dBm/ 38.16MHz | -52.86 |
| Propagation Condition |  |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled within BWoccupied. Note 3 SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. Note 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |

Table A.6.5.6.1.1.1-4: NR Cell specific test parameters for NR SCell for DL BWP switch in SA

| Parameter |  | Unit | Cell2 |
| --- | --- | --- | --- |
| Frequency Range |  |  | FR1 |
| Duplex mode | ConfigSCell 1 |  | FDD |
|  | ConfigSCell 2,3 |  | TDD |
| TDD configuration | ConfigSCell 1 |  | Not Applicable |
|  | ConfigSCell 2 |  | TDDConf.1.1 |
|  | ConfigSCell 3 |  | TDDConf.1.2 |
| BWchannel | ConfigSCell 1,2 |  | Note 7 |
|  | ConfigSCell 3 |  | Note 7 |
| BWoccupied | ConfigSCell 1,2 | RB | 52 Note 5 |
|  | ConfigSCell 3 |  | 106 Note 6 |
| Active BWP ID |  |  | 1, 2 |
| Initial DL BWP Configuration |  |  | DLBWP.0.2Note4 |
| Initial UL BWP Configuration |  |  | N.A. |
| Active DL BWP-0 Configuration |  |  | N.A. |
| Active DL BWP-1 Configuration |  |  | DLBWP.1.1Note4 |
| Active DL BWP-2 Configuration |  |  | DLBWP.1.3Note4 |
| Active UL BWP-0 Configuration |  |  | N.A. |
| Active UL BWP-1 Configuration |  |  | N.A. |
| Active UL BWP-2 Configuration |  |  | N.A. |
| PDSCH Reference | ConfigSCell 1 |  | SR.1.1 FDD |
| measurement channel | ConfigSCell 2 |  | SR.1.1 TDD |
|  | ConfigSCell 3 |  | SR.2.1 TDD |
| RMSI CORESET | ConfigSCell 1 |  | CR.1.1 FDD |
| parameters | ConfigSCell 2 |  | CR.1.1 TDD |
|  | ConfigSCell 3 |  | CR.2.1 TDD |
| Dedicated CORESET | ConfigSCell 1 |  | CCR.1.2 FDD |
| parameters | ConfigSCell 2 |  | CCR.1.2 TDD |
|  | ConfigSCell 3 |  | CCR.2.4 TDD |
| TRS Configuration | ConfigSCell 1 |  | TRS.1.1 FDD |
|  | ConfigSCell 2 |  | TRS.1.1 TDD |
|  | ConfigSCell 3 |  | TRS.1.2 TDD |
| OCNG Patterns | ConfigSCell 1,2 |  | OP.1 Note 5 |
|  | ConfigSCell 3 |  | OP.1 Note 6 |
| SSB Configuration | ConfigSCell 1,2 |  | SSB.1 FR1 |
|  | ConfigSCell 3 |  | SSB.2 FR1 |
| SMTC Configuration |  |  | SMTC.1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| NocNote 2 | ConfigSCell 1,2 | dBm/SCS | -104 |
|  | ConfigSCell 3 |  | -101 |
| NocNote 2 |  | dBm/15KHz | -104 |
| SS-RSRP Note 3 | ConfigSCell 1,2 | dBm/SCS | -87 |
|  | ConfigSCell 3 |  | -84 |
| Ês/Iot |  | dB | 17 |
| Ês/Noc |  | dB | 17 |
| IoNote3 | ConfigSCell 1,2 | dBm/ 9.36MHz | -58.96 |
|  | ConfigSCell 3 | dBm/ 38.16MHz | -52.86 |
| Propagation Condition |  |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled within BWoccupied. Note 3 SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. Note 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |

A.6.5.6.1.1.2	Test Requirements

During T1, the UE shall start to send the ACK/NACK for SCell on PCell from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+k1).

During T3, the UE shall start to send the ACK/NACK for SCell on PCell from the first UL slot that occurs after the beginning of DL slot (j+TBWPswitchDelay+k1).

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [7].

Depending on UE capability bwp-SwitchingDelay [2], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in Table 8.6.2-1.

All of the above test requirements shall be fulfilled in order for the observed SCell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

During T1 and T3, the start time of PCell interruption during SCell active BWP switch shall not happen outside the BWP switch delay.

The interruption of PCell shall not be longer than the interruption duration specified for active BWP switch in clause 8.2.2.2.5.

All of the above test requirements shall be fulfilled in order for the observed SCell active BWP switch interruption to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	During T1, T3 if there are no uplink resources for reporting the ACK/NACK in the first DL slot that occurs after the beginning of DL slot (i+ TBWPswitchDelay+k1), (j+ TBWPswitchDelay+k1), then the UE shall use the next available uplink resource for reporting the corresponding ACK/NACK.

##### A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA

A.6.5.6.1.2.1	Test Purpose and Environment

The purpose of this test is to verify the DL BWP switch delay requirement defined in clause 8.6.

The supported test configurations are shown in Table A.6.5.6.1.2.1-1. The test scenario comprises of one cell (Cell 1) as given in Table A.6.5.6.1.2.1-2. Cell-specific parameters of the cell are specified in Table A.6.5.6.1.2.1-3 below.

PDCCHs indicating new transmissions shall be sent continuously on Cell 1 to ensure that the UE will have ACK/NACK sending.

Before the test starts,

-	UE is connected to Cell 1 on radio channel 1.

-	UE is configured with 2 different UE-specific downlink bandwidth parts, BWP-1 and BWP-2 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1.

-	UE is configured with a bwp-InactivityTimer timer value for Cell1.

The cell has constant signal levels throughout the test.

The test consists of 3 successive time periods, with durations of T1, T2, and T3, respectively.

During T1,

Time period T1 starts when a DCI format 1_1 command for DL BWP switch, sent from the test equipment to the UE, is received at the UE side in Cell1’s slot # denoted i. The UE shall switch its bandwidth part from BWP-1 to BWP-2.

The UE shall be able to receive PDSCH on the first DL slot that occurs after the beginning of Cell1’s DL slot (i+TBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK for the Cell1 no later than the first UL slot that occurs after the beginning of slot (i+TBWPswitchDelay+k1). The UE shall be continuously scheduled on Cell1’s BWP-2 starting from the first DL slot that occurs after the beginning of slot (i+TBWPswitchDelay).

During T2, the test equipment won’t transmit DCI format for PDSCH reception on Cell1.

During T3,

The time period T3 starts from the slot #j, where j is the first slot of the subframe immediately after bwp-InactivityTimer timer expires. The UE shall switch its bandwidth part from BWP-2 back to the default bandwidth part – BWP-1.

The UE shall be able to receive PDSCH on the first DL slot that occurs after the beginning of Cell1’s slot (j+TBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK for the Cell1 at latest on the first UL slot that occurs after the beginning of slot (j+TBWPswitchDelay+k1). The UE shall be continuously scheduled on Cell1’s BWP-1 starting from the first DL slot that occurs after the beginning of slot (j+TBWPswitchDelay).

The test equipment verifies the DL BWP switch time by counting the slots from the time when the BWP switch command is received or bwp-InactivityTimer timer expires till an ACK/NACK is received.

Table A.6.5.6.1.2.1-1: DL BWP switch supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: A UE which fulfils the requirements in test case A.6.5.6.1.1 can skip the test cases in A.6.5.6.1.2. |  |

Table A.6.5.6.1.2.1-2: General test parameters for DL BWP switch in SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active Cell |  | Cell 1 | Cell1 on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| bwp-InactivityTimer | ms | 200 |  |
| T1 | s | 0.2 |  |
| T2 | s | 0.2 |  |
| T3 | s | 0.2 |  |

Table A.6.5.6.1.2.1-3: NR Cell specific test parameters for DL BWP switch in SA

| Parameter |  |  |  | Unit | Cell 1 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  |  |  | FR1 |
| Duplex mode |  |  | Config 1 |  | FDD |
|  |  |  | Config 2,3 |  | TDD |
| TDD configuration |  |  | Config 1 |  | Not Applicable |
|  |  |  | Config 2 |  | TDDConf.1.1 |
|  |  |  | Config 3 |  | TDDConf.2.1 |
| BWchannel |  |  | Config 1 |  | 10 MHz: NRB,c = 52 |
|  |  |  | Config 2 |  | 10 MHz: NRB,c = 52 |
|  |  |  | Config 3 |  | 40 MHz: NRB,c = 106 |
| Active BWP ID |  |  |  |  | 1, 2 |
| Initial DL BWP Configuration |  |  | Config 1,2,3 |  | DLBWP.0.2 Note 4 |
| Active DL BWP-1 Configuration |  |  | Config 1,2,3 |  | DLBWP.1.1 Note 4 |
| Active DL BWP-2 Configuration |  |  | Config 1,2,3 |  | DLBWP.1.3 Note 4 |
| Initial UL BWP Configuration |  |  | Config 1,2,3 |  | ULBWP.0.2 Note 4 |
| Active UL BWP-1 Configuration |  |  | Config 1,2,3 |  | ULBWP.1.1 Note 4 |
| Active UL BWP-2 Configuration |  |  | Config 1 |  | N/A |
|  |  |  | Config 2,3 |  | ULBWP.1.3 Note 4 |
| PDSCH Reference measurement channel |  |  | Config 1 |  | SR.1.1 FDD |
|  |  |  | Config 2 |  | SR.1.1 TDD |
|  |  |  | Config 3 |  | SR.2.1 TDD |
| RMSI CORESET parameters |  |  | Config 1 |  | CR.1.1 FDD |
|  |  |  | Config 2 |  | CR.1.1 TDD |
|  |  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET parameters |  |  | Config 1 |  | CCR.1.2 FDD |
|  |  |  | Config 2 |  | CCR.1.2 TDD |
|  |  |  | Config 3 |  | CCR.2.4 TDD |
| OCNG Patterns |  |  |  |  | OP.1 |
| SSB Configuration |  |  | Config 1,2 |  | SSB.1 FR1 |
|  |  |  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration |  |  |  |  | SMTC.1 |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 1x2 Low |
| TRS Configuration |  | Config 1,4 |  |  | TRS.1.1 FDD |
|  |  | Config 2,5 |  |  | TRS.1.1 TDD |
|  |  | Config 3,6 |  |  | TRS.1.2 TDD |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| NocNote 2 | Config 1,2 |  |  | dBm/SCS | -104 |
|  | Config 3 |  |  |  | -101 |
| NocNote 2 |  |  |  | dBm/15kHz | -104 |
| SS-RSRP Note 3 | Config 1,2 |  |  | dBm/SCS | -87 |
|  | Config 3 |  |  |  | -84 |
| Ês/Iot |  |  |  | dB | 17 |
| Ês/Noc |  |  |  | dB | 17 |
| IoNote3 |  |  | Config 1,2 | dBm/ 9.36MHz | -58.96 |
|  |  |  | Config 3 | dBm/ 38.16MHz | -52.86 |
| Propagation Condition |  |  |  |  | AWGN |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. |  |  |  |  |  |

A.6.5.6.1.2.2	Test Requirements

During T1, the UE shall start to send the ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+k1).

During T3, the UE shall start to send the ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot (j+TBWPswitchDelay+k1).

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [7].

Depending on UE capability bwp-SwitchingDelay [2], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in Table 8.6.2-1.

All of the above test requirements shall be fulfilled in order for the observed Cell1 active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	During T1, T3 if there are no uplink resources for reporting the ACK/NACK in the first UL slot that occurs after beginning of DL slot (i+TBWPswitchDelay+k1), (j+TBWPswitchDelay+k1), then the UE shall use the next available uplink resource for reporting the corresponding ACK/NACK.

#### A.6.5.6.2	RRC-based Active BWP Switch

##### A.6.5.6.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA

A.6.5.6.2.1.1	Test Purpose and Environment

The purpose of this test is to verify the DL BWP switch delay requirement for RRC-based BWP switch defined in clause 8.6.

The supported test configurations are shown in Table A.6.5.6.2.1.1-1. The test scenario comprises of one Cell (Cell 1) as given in Table A.6.5.6.2.1.1-2. Cell-specific parameters of Cell are specified in Table A.6.5.6.2.1.1-3 below.

PDCCHs indicating new transmissions shall be sent continuously on Cell 1 to ensure that the UE will have ACK/NACK sending.

Before the test starts,

-	UE is connected to Cell 1 on radio channel 1.

-	UE has bandwidth part BWP-1 in its RRC-configuration for Cell 1.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 of initial condition in Cell 1.

All cells have constant signal levels throughout the test.

The test consists of 1 time period, with duration of T1.

During T1,

Time period T1 starts when a RRCReconfiguration with updated bandwidth part configuration, sent from the test equipment to the UE, is completely received at the UE side in PCell’s slot # denoted i. The UE shall reconfigure its bandwidth part with the updated bandwidth part BWP-1 of final condition.

The UE shall be able to receive PDSCH on PCell from the first DL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length as defined in clause 8.6.3 and starts to report valid ACK/NACK for the PCell from the first UL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length+k1 on BWP-1 of final condition. The UE shall be continuously scheduled on PCell’s BWP-1 of final condition starting from the first DL slot right after slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length.

TRRCprocessingDelay and TBWPswitchDelayRRC are defined in clause 8.6.3.

The test equipment verifies the DL BWP switch time in Cell by counting the time from the time when the RRC Reconfiguration message including updated BWP configuration is sent till the time when a vaild ACK/NACK is received is received.

Table A.6.5.6.2.1.1-1: DL BWP switch supported test configurations in SA scenario

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.5.6.2.1.1-2: General test parameters for DL BWP switch in SA scenario

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active Cell |  | Cell 1 | Cell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| T1 | s | 0.2 |  |

Table A.6.5.6.2.1.1-3: NR Cell specific test parameters for DL BWP switch in SA scenario

| Parameter |  |  |  | Unit | Cell 1 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  |  |  | FR1 |
| Duplex mode |  | Config 1 |  |  | FDD |
|  |  | Config 2,3 |  |  | TDD |
| TDD configuration |  | Config 1 |  |  | Not Applicable |
|  |  | Config 2 |  |  | TDDConf.1.1 |
|  |  | Config 3 |  |  | TDDConf.2.1 |
| BWchannel |  | Config 1 |  |  | 10 MHz: NRB,c = 52 |
|  |  | Config 2 |  |  | 10 MHz: NRB,c = 52 |
|  |  | Config 3 |  |  | 40 MHz: NRB,c = 106 |
| Active BWP ID |  |  |  |  | 1 |
| Initial DL BWP Configuration |  | Config 1,2, 3 |  |  | DLBWP.0.2 |
| Initial UL BWP Configuration |  | Config 1,2, 3 |  |  | ULBWP.0.2 |
| Initial Condition | Active DL BWP-1 Configuration | Config 1, 2, 3 |  |  | DLBWP.1.3 |
|  | Active UL BWP-1 Configuration | Config 1, 2, 3 |  |  | ULBWP.1.3 |
| Final Condition | Active DL BWP-1 Configuration | Config 1, 2, 3 |  |  | DLBWP.1.1 |
|  | Active UL BWP-1 Configuration | Config 1, 2, 3 |  |  | ULBWP.1.1 |
| PDSCH Reference measurement channel |  | Config 1 |  |  | SR.1.1 FDD |
|  |  | Config 2 |  |  | SR.1.1 TDD |
|  |  | Config 3 |  |  | SR2.1 TDD |
| RMSI CORESET parameters |  | Config 1 |  |  | CR.1.1 FDD |
|  |  | Config 2 |  |  | CR.1.1 TDD |
|  |  | Config 3 |  |  | CR2.1 TDD |
| Dedicated CORESET parameters |  | Config 1 |  |  | CCR.1.2 FDD |
|  |  | Config 2 |  |  | CCR.1.2 TDD |
|  |  | Config 3 |  |  | CCR.2.4 TDD |
| OCNG Patterns |  |  |  |  | OP.1 |
| SSB Configuration |  | Config 1,2 |  |  | SSB.1 FR1 |
|  |  | Config 3 |  |  | SSB.2 FR1 |
| SMTC Configuration |  |  |  |  | SMTC.1 |
| TRS Configuration |  |  | Config 1 |  | TRS.1.1 FDD |
|  |  |  | Config 2 |  | TRS.1.1 TDD |
|  |  |  | Config 3 |  | TRS.1.2 TDD |
| Antenna Configuration |  |  |  |  | 1x2 Low |
| Propagation Condition |  |  |  |  | AWGN |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS(Note 1) |  |  |  |  |  |
| NocNote 2 |  | Config 1,2 |  | dBm/SCS | -104 |
|  |  | Config 3 |  |  | -101 |
| SS-RSRP Note 3 |  | Config 1,2 |  | dBm/SCS | -87 |
|  |  | Config 3 |  |  | -84 |
| Ês/Iot |  |  |  | dB | 17 |
| Ês/Noc |  |  |  | dB | 17 |
| IoNote3 |  | Config 1,2 |  | dBm/ 9.36MHz | -58.96 |
|  |  | Config 3 |  | dBm/ 38.16MHz | -52.86 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. |  |  |  |  |  |

A.6.5.6.2.1.2	Test Requirements

During T1, the UE shall be ready for the reception of uplink grant for the Cell from the first DL slot that occurs right after the begining of slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length and starts to report valid ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length+k1.

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [7].

All of the above test requirements shall be fulfilled in order for the observed Cell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs

##### A.6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA

###### A.6.5.6.3.1.1	Test Purpose and Environment

The purpose of this test is to verify requirements on the DL BWP switch delay on multiple CCs and interruption requirement for NR victim cell, both defined in clause 8.6.

The supported test configurations for NR PCell are shown in Table A.6.5.6.3.1.1-1 below. The supported test configurations for NR SCells are shown in Table A.6.5.6.3.1.1-1A below. Test configuration for NR PCell and test configuration for NR SCells are chosen independently. Test configuration for two NR SCells are chosen independently. The test scenario comprises of one NR PCell (Cell 1) and two NR SCells (Cell 2 and Cell 3) as given in Table A.6.5.6.3.1.1-2. NR Cell-specific parameters are specified in Table A.6.5.6.3.1.1-3, Table A.6.5.6.3.1.1-4 and Table A.6.5.6.3.1.1-5 below.

PDCCHs indicating new transmissions shall be sent continuously on PCell (Cell 1) and SCell (Cell 2) to ensure that the UE would have ACK/NACK sending except for the time duration when BWP is switching on Cell 1 and the time duration of T2.

PDCCHs indicating new transmissions shall be sent continuously on SCell (Cell 3) to ensure that the UE will have ACK/NACK sending.Before the test starts,

-	UE is connected to PCell (Cell 1) on radio channel 1 (PCC), and SCell (Cell 2) on radio channel 2 (SCC) and SCell (Cell 3) on radio channel 3(SCC).

-	UE is configured with 2 different UE-specific downlink bandwidth parts for PCell and SCell (Cell 2), BWP-1 and BWP-2, in Cell 1 and Cell 2 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

-	UE is configured with a single UE-specific downlink bandwidth part, BWP-0, for SCell (Cell 3). BWP-0 includes the bandwidth of the initial DL BWP and SSB.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 in PCell and SCell (Cell 2).

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-0 in SCell (Cell 3).

-	UE is configured with a bwp-InactivityTimer timer value for PCell and SCell (Cell 2).

All cells have constant signal levels throughout the test.

The test consists of 3 successive time periods, with durations of T1, T2, and T3, respectively.

During T1,

Time period T1 starts when a DCI format 1_1 command for both PCell and SCell (Cell 2) DL BWP switch, sent from the test equipment to the UE, is received at the UE side in both PCell’s and SCell’s slot # denoted i. The UE shall switch its bandwidth part from BWP-1 to BWP-2 at both PCell and SCell (Cell 2).

The UE shall be able to receive PDSCH no later than the first DL slot that occurs after the beginning of PCell’s and SCell (Cell 2)’s DL slot (i+ TMultipleBWPswitchDelay) as defined in clause 8.6.2A.1 and starts to report valid ACK/NACK for the both PCell and SCell (Cell 2) no later than the first UL slot that occurs after the beginning of slot (i+ TMultipleBWPswitchDelay +k1). The UE shall be continuously scheduled on both PCell’s and SCell (Cell 2)’s BWP-2 no later than the first DL slot that occurs after the beginning of slot (i+ TMultipleBWPswitchDelay).

The starting time of SCell (Cell 3) interruption due to BWP switch on PCell and SCell (Cell 2) shall occur within the BWP switch delay.

During T2, the test equipment won’t transmit DCI format for PDSCH reception on PCell (Cell 1) and SCell (Cell 2).

During T3,

The time period T3 starts from the slot #j, where j is the first slot of the subframe immediately after bwp-InactivityTimer timer expires. The UE should switch its bandwidth part from BWP-2 back to the default bandwidth part – BWP-1 on both PCell and SCell (Cell 2).

The UE shall be able to receive PDSCH no later than the first DL slot that occurs after the beginning of PCell’s an SCell (Cell 2)’s slot (j+ TMultipleBWPswitchDelay) as defined in clause 8.6.2A.1 and starts to report valid ACK/NACK for the PCell and SCell (Cell 2) no later than the first UL slot that occurs after the beginning of slot (j+ TMultipleBWPswitchDelay +k1). The UE shall be continuously scheduled on PCell’s and SCell (Cell 2)’s BWP-1 no later than the first DL slot that occurs after the beginning of slot (j+ TMultipleBWPswitchDelay).

The starting time of SCell (Cell 3) interruption due to BWP switch of PCell shall occur within the BWP switch delay.

The test equipment verifies the DL BWP switch time in PCell and SCells by counting the slots from the time when the BWP switch command is received or bwp-InactivityTimer timer expires till an ACK/NACK is received.

The test equipment verifies that potential interruption to SCell (Cell 3) is carried out in the correct time span by monitoring ACK/NACK sent in SCell (Cell 3) during BWP switch of PCell, respectively.

Table A.6.5.6.3.1.1-1: DL BWP switch supported test configurations for NR PCell

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration |  |

Table A.6.5.6.3.1.1-1A: DL BWP switch supported test configurations for NR SCells

| ConfigSCell | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration Note 3: Test configurations of two NR SCells are selected independently. |  |

Table A.6.5.6.3.1.1-2: General test parameters for DL BWP switch in SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1, 2 | Two NR radio channels are used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active SCell |  | Cell 2; Cell 3 | SCell on RF channel number 2 and number 3. |
| CP length |  | Normal |  |
| DRX |  | OFF | For both PCell and SCells (Cell 2 and Cell 3) |
| bwp-InactivityTimer | ms | 200 |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for Cell 1 on SCC. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for Cell 2 on SCC. |
| Cell2 and Cell 3 timing offset to cell1 | s | 3 | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. |
| T1 | s | [0.2] |  |
| T2 | s | [0.2] |  |
| T3 | s | [0.2] |  |

Table A.6.5.6.3.1.1-3: NR Cell specific test parameters for NR PCell for DL BWP switch in SA

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| Frequency Range |  |  | FR1 |
| Duplex mode | Config 1 |  | FDD |
|  | Config 2,3 |  | TDD |
| TDD configuration | Config 1 |  | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.2.1 |
| BWchannel | Config 1,2 |  | 10 MHz: NRB,c = 52 |
|  | Config 3 |  | 40 MHz: NRB,c = 106 |
| Active BWP ID |  |  | 1, 2 |
| Initial DL BWP Configuration |  |  | DLBWP.0.2Note4 |
| Initial UL BWP Configuration |  |  | ULBWP.0.2Note4 |
| Active DL BWP-0 Configuration |  |  | N.A. |
| Active DL BWP-1 Configuration |  |  | DLBWP.1.1Note4 |
| Active DL BWP-2 Configuration |  |  | DLBWP.1.3Note4 |
| Active UL BWP-0 Configuration |  |  | N.A. |
| Active UL BWP-1 Configuration |  |  | ULBWP.1.1Note4 |
| Active UL BWP-2 Configuration |  |  | ULBWP.1.3Note4 |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |
|  | Config 2 |  | SR.1.1 TDD |
|  | Config 3 |  | SR.2.1 TDD |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD |
|  | Config 2 |  | CCR.1.1 TDD |
|  | Config 3 |  | CCR.2.1 TDD |
| OCNG Patterns |  |  | OP.1 |
| SSB Configuration | Config 1,2 |  | SSB.1 FR1 |
|  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration |  |  | SMTC.1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |
| NocNote 2 | Config 1,2 | dBm/SCS | -104 |
|  | Config 3 |  | -101 |
| NocNote 2 |  | dBm/15KHz | -104 |
| SS-RSRP Note 3 | Config 1,2 | dBm/SCS | -87 |
|  | Config 3 |  | -84 |
| Ês/Iot |  | dB | 17 |
| Ês/Noc |  | dB | 17 |
| IoNote3 | Config 1,2 | dBm/ 9.36MHz | -58.96 |
|  | Config 3 | dBm/ 38.16MHz | -52.86 |
| Propagation Condition |  |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3 SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. |  |  |  |

Table A.6.5.6.3.1.1-4: NR Cell specific test parameters for SCell (NR Cell 2) for DL BWP switch in SA

| Parameter |  | Unit | Cell 2 |
| --- | --- | --- | --- |
| Frequency Range |  |  | FR1 |
| Duplex mode | ConfigSCell1 1 |  | FDD |
|  | ConfigSCell1 2,3 |  | TDD |
| TDD configuration | ConfigSCell1 1 |  | Not Applicable |
|  | ConfigSCell1 2 |  | TDDConf.1.1 |
|  | ConfigSCell1 3 |  | TDDConf.2.1 |
| BWchannel | ConfigSCell1 1,2 |  | 10 MHz: NRB,c = 52 |
|  | ConfigSCell1 3 |  | 40 MHz: NRB,c = 106 |
| Active BWP ID |  |  | 1, 2 |
| Initial DL BWP Configuration |  |  | DLBWP.0.2Note4 |
| Initial UL BWP Configuration |  |  | ULBWP.0.2Note4 |
| Active DL BWP-0 Configuration |  |  | N.A. |
| Active DL BWP-1 Configuration |  |  | DLBWP.1.1Note4 |
| Active DL BWP-2 Configuration |  |  | DLBWP.1.3Note4 |
| Active UL BWP-0 Configuration |  |  | N.A. |
| Active UL BWP-1 Configuration |  |  | ULBWP.1.1Note4 |
| Active UL BWP-2 Configuration |  |  | ULBWP.1.3Note4 |
| PDSCH Reference measurement channel | ConfigSCell1 1 |  | SR.1.1 FDD |
|  | ConfigSCell1 2 |  | SR.1.1 TDD |
|  | ConfigSCell1 3 |  | SR.2.1 TDD |
| RMSI CORESET parameters | ConfigSCell1 1 |  | CR.1.1 FDD |
|  | ConfigSCell1 2 |  | CR.1.1 TDD |
|  | ConfigSCell1 3 |  | CR.2.1 TDD |
| Dedicated CORESET parameters | ConfigSCell1 1 |  | CCR.1.1 FDD |
|  | ConfigSCell1 2 |  | CCR.1.1 TDD |
|  | ConfigSCell1 3 |  | CCR.2.1 TDD |
| OCNG Patterns |  |  | OP.1 |
| SSB Configuration | ConfigSCell1 1,2 |  | SSB.1 FR1 |
|  | ConfigSCell1 3 |  | SSB.2 FR1 |
| SMTC Configuration |  |  | SMTC.1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |
| NocNote 2 | ConfigSCell1 1,2 | dBm/SCS | -104 |
|  | ConfigSCell1 3 |  | -101 |
| NocNote 2 |  | dBm/15KHz | -104 |
| SS-RSRP Note 3 | ConfigSCell1 1,2 | dBm/SCS | -87 |
|  | ConfigSCell1 3 |  | -84 |
| Ês/Iot |  | dB | 17 |
| Ês/Noc |  | dB | 17 |
| IoNote3 | ConfigSCell1 1,2 | dBm/ 9.36MHz | -58.96 |
|  | ConfigSCell1 3 | dBm/ 38.16MHz | -52.86 |
| Propagation Condition |  |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3 SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. |  |  |  |

Table A.6.5.6.3.1.1-5: NR Cell specific test parameters for SCell (NR Cell 3) for DL BWP switch in SA

| Parameter |  |  | Unit | Cell 3 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  |  | FR1 |
| Duplex mode |  | ConfigSCell2 1 |  | FDD |
|  |  | ConfigSCell2 2,3 |  | TDD |
| TDD configuration |  | ConfigSCell2 1 |  | Not Applicable |
|  |  | ConfigSCell2 2 |  | TDDConf.1.1 |
|  |  | ConfigSCell2 3 |  | TDDConf.2.1 |
| BWchannel |  | ConfigSCell2 1,2 |  | 10 MHz: NRB,c = 52 |
|  |  | ConfigSCell2 3 |  | 40 MHz: NRB,c = 106 |
| Active BWP ID |  |  |  | 0 |
| Initial DL BWP Configuration |  |  |  | DLBWP.0.2Note4 |
| Initial UL BWP Configuration |  |  |  | ULBWP.0.2Note4 |
| Active DL BWP-0 Configuration |  |  |  | DLBWP.0.2Note4 |
| Active DL BWP-1 Configuration |  |  |  | N.A. |
| Active DL BWP-2 Configuration |  |  |  | N.A. |
| Active UL BWP-0 Configuration |  |  |  | ULBWP.0.2Note4 |
| Active UL BWP-1 Configuration |  |  |  | N.A. |
| Active UL BWP-2 Configuration |  |  |  | N.A. |
| PDSCH Reference measurement channel |  | ConfigSCell2 1 |  | SR.1.1 FDD |
|  |  | ConfigSCell2 2 |  | SR.1.1 TDD |
|  |  | ConfigSCell2 3 |  | SR.2.1 TDD |
| RMSI CORESET parameters |  | ConfigSCell2 1 |  | CR.1.1 FDD |
|  |  | ConfigSCell2 2 |  | CR.1.1 TDD |
|  |  | ConfigSCell2 3 |  | CR.2.1 TDD |
| Dedicated CORESET parameters |  | ConfigSCell2 1 |  | CCR.1.1 FDD |
|  |  | ConfigSCell2 2 |  | CCR.1.1 TDD |
|  |  | ConfigSCell2 3 |  | CCR.2.1 TDD |
| OCNG Patterns |  |  |  | OP.1 |
| SSB Configuration |  | ConfigSCell2 1,2 |  | SSB.1 FR1 |
|  |  | ConfigSCell2 3 |  | SSB.2 FR1 |
| SMTC Configuration |  |  |  | SMTC.1 |
| Correlation Matrix and Antenna Configuration |  |  |  | 1x2 Low |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| NocNote 2 | ConfigSCell2 1,2 |  | dBm/SCS | -104 |
|  | ConfigSCell2 3 |  |  | -101 |
| NocNote 2 |  |  | dBm/15KHz | -104 |
| SS-RSRP Note 3 | ConfigSCell2 1,2 |  | dBm/SCS | -87 |
|  | ConfigSCell2 3 |  |  | -84 |
| Ês/Iot |  |  | dB | 17 |
| Ês/Noc |  |  | dB | 17 |
| IoNote3 |  | ConfigSCell2 1,2 | dBm/ 9.36MHz | -58.96 |
|  |  | ConfigSCell2 3 | dBm/ 38.16MHz | -52.86 |
| Propagation Condition |  |  |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3 SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. |  |  |  |  |

###### A.6.5.6.3.1.2	Test Requirements

During T1, the UE shall start to send the ACK/NACK for both PCell and SCell (Cell 2) from the first UL slot that occurs after the beginning of DL slot (i+ TMultipleBWPswitchDelay +k1).

During T3, the UE shall start to send the ACK/NACK for both PCell and SCell (Cell 2) from the first UL slot that occurs after the beginning of DL slot (j+ TMultipleBWPswitchDelay +k1).

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [7].

Depending on UE capability bwp-SwitchingDelay [2], UE shall finish BWP switch within the time duration TMultipleBWPswitchDelay defined in 8.6.2A.1.

All of the above test requirements shall be fulfilled in order for the observed Cell1 and Cell2 active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

During T1 and T3, the start time of SCell (Cell 3) interruption during PCell and SCell (Cell 2) active BWP switch shall not happen outside the BWP switch delay.

The interruption of SCell (Cell 3) shall not be longer than the interruption duration specified for active BWP switch in clause 8.2.2.2.5.

All of the above test requirements shall be fulfilled in order for the observed PCell and SCell (Cell 2) active BWP switch interruption to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	During T1, T3 if there are no uplink resources for reporting the ACK/NACK in the first UL slot that occurs after beginning of DL slot (i+ TMultipleBWPswitchDelay +k1), (j+ TMultipleBWPswitchDelay +k1), then the UE shall use the next available uplink resource for reporting the corresponding ACK/NACK.

#### A.6.5.6.4	SCell dormancy switch

##### A.6.5.6.4.1	NR FR1 PCell SCell dormancy switch of single FR1 SCell outside active time

###### A.6.5.6.4.1.1	Test Purpose and Environment

The purpose of this test is to verify the SCell dormancy switch delay requirements defined in clause 8.6 when the UE is triggered to switch between dormancy to non-dormancy and non-dormancy to dormancy outside the DRX active time. Further the test purpose is to verify the interruption rate on other serving cells when the UE performing CSI and RRM measurements on dormant SCell(s) as defined in clause 8.2.2.2.12 and also to verify the interruption requirement on other active serving cell defined in clause 8.2.2.2.5.

In the test scenario UE is connected to one PCell (Cell 1) in FR1 and one SCell in FR1. In the test the SCell is switched from non-dormancy to dormancy, and vice versa, at a point in time before start of onDuration. The UE is configured to monitor PDCCH for DCI format 2_6 at ps-Offset before the start of onDuration. Two tests are specified, where a UE that only supports triggering within the first three OFDM symbols of a slot shall undergo Test1 only, and a UE that supports triggering also in remaining OFDM symbols of a slot shall undergo both Test1 and Test2. In the tested scenario, ps-Offset is selected to correspond to the dormancy switching time specified in clause 8.6.

The supported test configurations for NR PCell are shown in Table A.6.5.6.4.1.1-1, The supported test configurations for NR SCell are shown in Table A.6.5.6.4.1.1-1A. Test configuration for NR PCell and test configuration for NR SCell are chosen independently. The general test parameters are given in Table A.6.5.6.4.1.1-2. NR Cell-specific parameters are specified in Table A.6.5.6.4.1.1-3 and Table A.6.5.6.4.1.1-4.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), and Cell 2 (SCell) on radio channel 2 (SCC).

-	UE is configured with 1 UE-specific downlink bandwidth parts the same as initial BWP for PCell, BWP-0 in Cell 1 before starting the test.

-	UE is configured with 2 different UE-specific downlink bandwidth parts for SCell, BWP-1 and BWP-2, in Cell 2 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-0 in PCell.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 in SCell.

-	UE is indicated in dormantBWP -Id that the active DL BWP is BWP-2 in the SCell.

-	UE is configured with DRX.

-	UE is configured to monitor DCI format 2_6, and to be active during onDuration even when no DCI format 2_6 is detected (ps-WakeUp).

All cells have constant signal levels throughout the test.

The test consists of 4 successive time periods, with durations of T1, T2, T3 and T4, respectively.

During T1,

Time period T1 starts when a DCI format 2_6 command intended for dormant BWP switch in a SCell from non-dormancy to dormancy, sent from the test equipment to the UE, is received at the UE side in PCell’s slot # denoted i (at ps-Offset before onDuration). Upon reception of the PDCCH indicating entering dormant BWP in PCell (i.e. through cross-carrier scheduling), UE shall switch the DL BWP-1 to DL BWP-2 in SCell, i.e., switching from non-dormant BWP to dormant BWP and the UE shall complete the switching before the start of onDuration.

The UE shall be able to receive PDCCH on PCell no later than the first DL slot that occurs after the beginning of PCell’s DL slot (i+ TdormantBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK on the PCell no later than the first UL slot that occurs after the beginning of slot (i+N) as defined in clause 10.3 in TS38.213. The UE shall be continuously scheduled on PCell’s BWP-0 no later than the first DL slot that occurs after the beginning of slot (i+ TdormantBWPswitchDelay).

The starting time of PCell (Cell 1) interruption due to dormancy switching on SCell shall occur within the dormant BWP switch delay, i.e. before start of onDuration.

The UE shall not transmit signals on SCell after the beginning of PCell’s DL slot (i+ TdormantBWPswitchDelay) as defined in clause 8.6. The UE shall not be scheduled on SCells BWP-1 no later than the first DL slot that occurs after the beginning of slot (i+ TdormantBWPswitchDelay).

Time period T2 starts when T1 is completed. During T2, the test equipment continues to schedule the UE continuously in PCell. The UE shall carry out CSI and RRM measurements on the dormant SCells. The UE shall report ACK/NACK in PCell in response to scheduled PDSCH, with the maximum loss of transmitted ACK/NACKs fulfilling the requirement in clause 8.2.2.2.12. The test equipment verifies that the loss of ACK/NACKs is no larger than 1.5%.

Time period T3 starts when T2 is completed. During T3, the test equipment does not schedule the UE, by which the inactivity timer expires and the UE stops monitoring PDCCH except for signalling using DCI format 2_6 at wake-up signalling occasions.

During T4,

Time period T4 starts when a DCI format 2_6 command for leaving dormant BWP in SCell, sent from the test equipment to the UE, is received at the UE side in PCell’s slot # denoted j (at ps-Offset before onDuration). Upon reception of the PDCCH indicating leaving dormant BWP in PCell (i.e. through cross-carrier scheduling), UE shall switch the DL BWP-2 to DL BWP-1 in SCell, i.e., switching from dormant BWP to non-dormant BWP.

The UE shall be able to receive PDSCH on PCell and SCell no later than the first DL slot that occurs after the beginning of PCell’s DL slot (j+ TdormantBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK on the PCell (for both PCell and SCell) no later than the first UL slot that occurs after the beginning of slot (j+N) as defined in clause 10.3 in TS 38.213. The UE shall be continuously scheduled on PCell’s BWP-0 no later than the first DL slot that occurs after the beginning of slot (j+ TdormantBWPswitchDelay).

The starting time of PCell (Cell 1) interruption due to dormancy switching on SCell shall occur within the dormant BWP switch delay.

The UE shall be ready to transmit signals on SCell no later than the first DL slot that occurs after the beginning of PCell’s DL slot (j+ TdormantBWPswitchDelay) as defined in clause 8.6. The UE shall be ready to continuously scheduled on SCell’s BWP-1 no later than the first DL slot that occurs after the beginning of slot (j+ TdormantBWPswitchDelay).

The test equipment verifies the DL dormant BWP switch time in SCell by counting the slots from the time when the dormant BWP switch command is received till an ACK/NACK on PCell is received.

The test equipment verifies that potential interruption to PCell is carried out in the correct time span by monitoring ACK/NACK sent in PCell during dormant BWP switch of SCell (i.e. before start of onDuration), respectively.

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration |  |

Table A.6.5.6.4.1.1-1A: SCell dormancy switch supported test configurations for NR SCell

| ConfigSCell | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration |  |

Table A.6.5.6.4.1.1-2: General test parameters for SCell dormancy switch in SA

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | 1, 2 |  | Two NR radio channels are used for this test |
| Active PCell |  | Cell 1 |  | PCell on RF channel number 1. |
| Active SCell |  | Cell 2 |  | SCell on RF channel number 2. |
| CP length |  | Normal |  |  |
| DRX |  | ON |  | For both PCell and SCell |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 |  | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 |  | Individual offset for cells on SCC. |
| Cell2 timing offset to cell1 | s | 3 |  | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. |
| OFDM symbol range in slot for transmission of DCI with dormancy indication |  | 0 – 2 | 3 – 11 |  |
| T1 | s | 0.2 |  |  |
| T2 | s | 0.2 |  |  |

Table A.6.5.6.4.1.1-3: NR Cell specific test parameters for NR PCell for SCell dormancy switch in SA

| Parameter |  |  | Unit | Cell 1 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  |  | FR1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2,3 |  | TDD |
| TDD configuration |  | Config 1 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| BWchannel |  | Config 1,2 |  | 10 MHz: NRB,c = 52 |
|  |  | Config 3 |  | 40 MHz: NRB,c = 106 |
| Active BWP ID |  |  |  | 0 |
| Initial DL BWP Configuration |  |  |  | DLBWP.0.2Note4 |
| Initial UL BWP Configuration |  |  |  | ULBWP.0.2Note4 |
| Active DL BWP-0 Configuration |  |  |  | DLBWP.0.2Note4 |
| Active DL BWP-1 Configuration |  |  |  | N.A. |
| Active DL BWP-2 Configuration |  |  |  | N.A. |
| Active UL BWP-0 Configuration |  |  |  | ULBWP.0.2Note4 |
| Active UL BWP-1 Configuration |  |  |  | N.A. |
| Active UL BWP-2 Configuration |  |  |  | N.A. |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |
|  |  | Config 2 |  | SR.1.1 TDD |
|  |  | Config 3 |  | SR.2.1 TDD |
| RMSI CORESET parameters |  | Config 1 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET parameters, Test 1 |  | Config 1 |  | CCR.1.1 FDD |
|  |  | Config 2 |  | CCR.1.1 TDD |
|  |  | Config 3 |  | CCR.2.1 TDD |
| Dedicated CORESET parameters, Test 2 |  | Config 1 |  | CCR.1.5 FDD |
|  |  | Config 2 |  | CCR.1.5 TDD |
|  |  | Config 3 |  | CCR.2.3 TDD |
| OCNG Patterns |  |  |  | OP.1 |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration |  |  |  | SMTC.1 |
| Correlation Matrix and Antenna Configuration |  |  |  | 1x2 Low |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| NocNote 2 | Config 1,2 |  | dBm/SCS | -104 |
|  | Config 3 |  |  | -101 |
| NocNote 2 |  |  | dBm/15KHz | -104 |
| SS-RSRP Note 3 | Config 1,2 |  | dBm/SCS | -87 |
|  | Config 3 |  |  | -84 |
| Ês/Iot |  |  | dB | 17 |
| Ês/Noc |  |  | dB | 17 |
| IoNote3 |  | Config 1,2 | dBm/ 9.36MHz | -58.96 |
|  |  | Config 3 | dBm/ 38.16MHz | -52.86 |
| Propagation Condition |  |  |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3 SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. |  |  |  |  |

Table A.6.5.6.4.1.1-4: NR Cell specific test parameters for NR SCell for SCell dormancy switch in SA

| Parameter |  |  | Unit | Cell2 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  |  | FR1 |
| Duplex mode |  | ConfigSCell 1 |  | FDD |
|  |  | ConfigSCell 2,3 |  | TDD |
| TDD configuration |  | ConfigSCell 1 |  | Not Applicable |
|  |  | ConfigSCell 2 |  | TDDConf.1.1 |
|  |  | ConfigSCell 3 |  | TDDConf.2.1 |
| BWchannel |  | ConfigSCell 1,2 |  | 10 MHz: NRB,c = 52 |
|  |  | ConfigSCell 3 |  | 40 MHz: NRB,c = 106 |
| Active BWP ID |  |  |  | 1 |
| Initial DL BWP Configuration |  |  |  | DLBWP.0.2Note4 |
| Initial UL BWP Configuration |  |  |  | ULBWP.0.2Note4 |
| Active DL BWP-0 Configuration |  |  |  | N.A. |
| Active DL BWP-1 Configuration |  |  |  | DLBWP.1.1Note4 |
| Active DL BWP-2 Configuration |  |  |  | DLBWP.1.3Note4 |
| Active UL BWP-0 Configuration |  |  |  | N.A. |
| Active UL BWP-1 Configuration |  |  |  | ULBWP.1.1Note4 |
| Active UL BWP-2 Configuration |  |  |  | ULBWP.1.3Note4 |
| PDSCH Reference measurement channel |  | ConfigSCell 1 |  | SR.1.1 FDD |
|  |  | ConfigSCell 2 |  | SR.1.1 TDD |
|  |  | ConfigSCell 3 |  | SR.2.1 TDD |
| RMSI CORESET parameters |  | ConfigSCell 1 |  | CR.1.1 FDD |
|  |  | ConfigSCell 2 |  | CR.1.1 TDD |
|  |  | ConfigSCell 3 |  | CR.2.1 TDD |
| Dedicated CORESET parameters, Test 1 |  | ConfigSCell 1 |  | CCR.1.1 FDD |
|  |  | ConfigSCell 2 |  | CCR.1.1 TDD |
|  |  | ConfigSCell 3 |  | CCR.2.1 TDD |
| Dedicated CORESET parameters, Test 2 |  | ConfigSCell 1 |  | CCR.1.1 FDD |
|  |  | ConfigSCell 2 |  | CCR.1.1 TDD |
|  |  | ConfigSCell 3 |  | CCR.2.3 TDD |
| OCNG Patterns |  |  |  | OP.1 |
| SSB Configuration |  | ConfigSCell 1,2 |  | SSB.1 FR1 |
|  |  | ConfigSCell 3 |  | SSB.2 FR1 |
| SMTC Configuration |  |  |  | SMTC.1 |
| Correlation Matrix and Antenna Configuration |  |  |  | 1x2 Low |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| NocNote 2 | ConfigSCell 1,2 |  | dBm/SCS | -104 |
|  | ConfigSCell 3 |  |  | -101 |
| NocNote 2 |  |  | dBm/15KHz | -104 |
| SS-RSRP Note 3 | ConfigSCell 1,2 |  | dBm/SCS | -87 |
|  | ConfigSCell 3 |  |  | -84 |
| Ês/Iot |  |  | dB | 17 |
| Ês/Noc |  |  | dB | 17 |
| IoNote3 |  | ConfigSCell 1,2 | dBm/ 9.36MHz | -58.96 |
|  |  | ConfigSCell 3 | dBm/ 38.16MHz | -52.86 |
| Propagation Condition |  |  |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3 SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. |  |  |  |  |

###### A.6.5.6.4.1.2	Test Requirements

During T1, the UE shall start to send the ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot (i+N) (i.e. from the start of onDuration).

During time period T2, the UE shall transmit ACK/NACKs in response to scheduling in PCell and the rate of missed ACK/NACKs shall be no more than 1.5%.

During T4, the UE shall start to send the ACK/NACK for PCell and SCell from the first UL slot that occurs after the beginning of DL slot (j+N) (i.e. from the start of onDuration).

Where, N is the timing that UE provide HARQ-ACK information in response to a detection of a DCI format 2_6 indicating SCell dormancy as specified in [3].

All of the above test requirements shall be fulfilled in order for the observed SCell dormant BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

During T1 and T4, the start time of PCell interruption during SCell dormant BWP switch shall not happen outside the dormant BWP switch delay.

The interruption of PCell shall not be longer than the interruption duration specified for dormant BWP switch in clause 8.6.

NOTE:	During T1, T4 if there are no uplink resources for reporting the ACK/NACK in the first DL slot that occurs after the beginning of DL slot (i+ N), (j+ N), then the UE shall use the next available uplink resource for reporting the corresponding ACK/NACK.

##### A.6.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR1 SCells inside active time

###### A.6.5.6.4.2.1		Test Purpose and Environment

The purpose of this test is to verify fulfillment of SCell dormancy switching delay requirements in clause 8.6.2A, requirements on interruptions due to SCell dormancy switching in clause 8.2.2.2.12.1, and requirements on interruptions due to CSI and RRM measurements on dormant SCells in clauses 8.2.2.2.12.2 and 8.2.2.2.12.3, respectively. In the tested scenario, the UE is connected to PCell and two SCells in FR1, and the SCells are switched from non-dormancy to dormancy, and back, during active time. Depending on UE capability on whether DCI for dormancy switching can be received also later than within the initial three OFDM symbols of a slot, the UE may have to undergo one or two sets of tests. A UE that only supports triggering during within the first three OFDM symbols of a slot shall only undergo Test1 and Test2, whereas a UE that supports triggering also in remaining OFDM symbols of a slot shall undergo Test1 through Test4.

The supported test configurations for NR PCell are provided in Table A.6.5.6.4.2.1-1 below. The supported test configurations for NR SCells are provided in Table A.6.5.6.4.2.1-1A below. Test configuration for NR PCell and test configuration for NR SCells are chosen independently. Test configurations for two NR SCells are chosen independently. General test parameters are provided in Table A.6.5.6.4.2.1-2, and cell-specific parameters are provided in Table A.6.5.6.4.2.1-3, Table A.6.5.6.4.2.1-4 and Table A.6.5.6.4.2.1-5 below.

The tests consist of three consecutive time periods T1, T2, and T3, respectively.

Three carriers are used in the test, each within FR1 and each with one cell. Cell 1 (PCell) is on RF channel 1 (PCC), Cell 2 (SCell1) is on RF channel 2 (SCC1), and Cell 3 (SCell2) is on RF channel 3 (SCC2). All three cells have constant signal levels throughout the test. The UE is continuously scheduled in PCell throughout the test.

Before the test starts,

-	UE is connected to Cell 1 (PCell), Cell 2 (SCell1) and Cell 3 (SCell2).

-	UE is configured with a single UE-specific downlink bandwidth part, BWP-0, for Cell 1. BWP-0 includes the bandwidth of the initial DL BWP and SSB.

-	UE is configured with one non-dormant and one dormant UE-specific downlink bandwidth part, BWP-0 and BWP-1, respectively, for Cell 2 and Cell 3. BWP-0 includes the bandwidth of the initial DL BWP and SSB.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP in Cell 1 is BWP-0.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP in Cell 2 is BWP-0.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP in Cell 3 is BWP-0.

-	UE is continuously scheduled in PCell, SCell1 and SCell2.

T1 starts at the point in time at which the UE receives a DCI with dormancy indication on PDCCH in PCell at the antenna connector, in a slot # denoted m, pertaining to dormancy indication for switching SCell1 and SCell2 from non-dormancy to dormancy. The UE shall complete switching of the SCells to dormancy by the end of slot m + ceil(TMultipleBWPswitchDelay/NR slot length) + 1 in Test1 and Test2, and slot m + ceil(TMultipleBWPswitchDelay/NR slot length) + 2 in Test3 and Test4, as specified in clause 8.6.2A. Any PCell interruptions due to the switching between non-dormant and dormant BWPs shall fulfill requirements in clause 8.2.2.2.12.1. The test equipment verifies that interruptions due to switching from non-dormancy to dormancy are within the requirements by analysing HARQ feedback transmitted in PCell for PCell.

During T2, the UE is carrying out CSI and RRM measurements on dormant SCell1 and SCell2. Any PCell interruptions due to CSI and RRM measurements shall fulfill requirements in clauses 8.2.2.2.12.2 and 8.2.2.2.12.3, respectively. The test equipment verifies that the interruptions are within the allowed percentages by counting ACK/NACKs in PCell. At the end of T2, the test equipment transmits a DCI with dormancy indication on PDCCH in PCell carrying a dormany indication for switching SCell1 and SCell2 from dormancy to non-dormancy.

T3 starts at the point in time at which the UE receives a DCI with dormancy indication on PDCCH in PCell at the antenna connector, in a slot # denoted n, pertaining to dormancy indication for switching SCell1 and SCell2 from dormancy to non-dormancy. The UE shall complete switching of the SCells to non-dormancy by the end of slot n + ceil(TMultipleBWPswitchDelay/NR slot length) + 1 in Test1 and Test2, and slot n + ceil(TMultipleBWPswitchDelay/NR slot length) + 2 in Test3 and test4, as specified in clause 8.6.2A. Any PCell interruptions due to the switching between dormant and non-dormant BWPs shall fulfill requirements in clause 8.2.2.2.12.1. The test equipment verifies that interruptions due to switching from dormancy to non-dormancy are within the requirements by analysing HARQ feedback transmitted in PCell for PCell. The test equipment verifies the switching delay by analysing HARQ feedback transmitted in PCell for SCells.

Table A.6.5.6.4.2.1-1: Supported test configurations for NR PCell

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration. |  |

Table A.6.5.6.4.2.1-1A: Supported test configurations for NR SCells

| ConfigSCell | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration. Note 3: Test configurations for two NR SCells are selected independently. |  |

Table A.6.5.6.4.2.1-2: General test parameters

| Parameter | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Test1 | Test2 | Test3 | Test4 |  |
| NR RF Channel Number |  | 1, 2, 3 |  |  |  | Three NR radio channels are used for this test |
| Active PCell |  | Cell 1 |  |  |  | Primary cell on NR RF channel number 1 in FR1 |
| SCell1 |  | Cell 2 |  |  |  | SCell1 on NR RF channel number 2 in FR1 |
| SCell2 |  | Cell 3 |  |  |  | SCell2 on NR RF channel number 3 in FR1 |
| CP length |  | Normal |  |  |  |  |
| DRX |  | OFF |  |  |  | Continuous monitoring of primary cell |
| CSI reporting periodicity, Non-dormant BWP | ms | 2 |  |  |  | CSI reporting periodicity for periodic reporting of CQI for PCell and non-dormant SCells |
| CSI reporting periodicity, Dormant BWP | ms | 40 |  |  |  | CSI reporting periodicity for periodic reporting of CQI for dormant SCells |
| Timing offset between Cell 1 and Cell 2 | ns | 0 |  |  |  |  |
| Timing offset between Cell 1 and Cell 3 | ns | 0 |  |  |  |  |
| Triggering DCI format |  | 1_1 | 0_1 | 1_1 | 0_1 | Triggering DCI format for triggering during active time |
| OFDM symbol range in slot for transmission of DCI with dormancy indication |  | 0 – 2 |  | 3 – 11 |  | Test1 and Test3 are based on that triggering DCI is received within the first three OFDM symbols of a slot.Test2 and Test4 are based on that the triggering DCI is received later than within the first three OFDM symbols of a slot. |
| T1 | s | 0.2 |  |  |  |  |
| T2 | s | 5 |  |  |  |  |
| T3 | s | 0.2 |  |  |  |  |

Table A.6.5.6.4.2.1-3: NR Cell specific test parameters for NR Pcell

| Parameter |  |  | Unit | Cell 1 |
| --- | --- | --- | --- | --- |
| Frequency range |  |  |  | FR1 |
| NR RF channel |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2,3 |  | TDD |
| TDD configuration |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| BWchannel |  | Config 1,2 | MHz | 10: NRB,c = 52 |
|  |  | Config 3 |  | 40: NRB,c = 106 |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |
| Downlink active non-dormant BWP-0 configuration |  |  |  | DLBWP.1.1 |
| Downlink active dormant BWP-1 configuration |  |  |  | --- |
| Uplink active BWP-0 configuration |  |  |  | ULBWP.1.1 |
| TCI state |  |  |  | TCI.State.0 |
| CSI-RS configuration for CSI reporting, Non-dormant BWP |  | Config 1 |  | CSI-RS.1.1 FDD |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |
| CSI-RS configuration for CSI reporting, Dormant BWP |  | Config 1 |  | --- |
|  |  | Config 2 |  |  |
|  |  | Config 3 |  |  |
| TRS Configuration |  | Config 1 |  | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |
|  |  | Config 2 |  | SR.1.1 TDD |
|  |  | Config 3 |  | SR.2.1 TDD |
| Dedicated CORESET parameters, Test 1,2 |  | Config 1 |  | CCR.1.1 FDD |
|  |  | Config 2 |  | CCR.1.1 TDD |
|  |  | Config 3 |  | CCR.2.1 TDD |
| Dedicated CORESET parameters, Test 3,4 |  | Config 1 |  | CCR.1.5 FDD |
|  |  | Config 2 |  | CCR.1.5 TDD |
|  |  | Config 3 |  | CCR.2.3 TDD |
| RMSI CORESET parameters |  | Config 1 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| OCNG Pattern |  |  |  | OP.1 |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.2 FR1 |
| SMTC configuration |  |  |  | SMTC.1 |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note1 |  |  |  |  |
| Noc Note2 | Config 1,2 |  | dBm/15kHz | -104 |
|  | Config 3 |  |  | -101 |
| Ês/Iot |  |  | dB | 17 |
| Ês/Noc |  |  | dB | 17 |
| SS-RSRP Note3 | Config 1,2 |  | dBm/SCS | -87 |
|  | Config 3 |  |  | -84 |
| Io Note3 | Config 1,2 |  | dBm/9.36 MHz | -59.0 |
|  | Config 3 |  | dBm/38.16 MHz | -52.9 |
| Propagation condition |  |  |  | AWGN |
| Correlation Matrix and Antenna Configuration |  |  |  | 1x2 Low |
| Note 1: OCNG shall be used such that the cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP, SCH_RP, and Io levels have been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |  |

Table A.6.5.6.4.2.1-4: NR Cell specific test parameters for NR SCell (Cell 2)

| Parameter |  | Unit | Cell 2 |
| --- | --- | --- | --- |
| Frequency range |  |  | FR1 |
| NR RF channel |  |  | 2 |
| Duplex mode | ConfigSCell1 1 |  | FDD |
|  | ConfigSCell1 2,3 |  | TDD |
| TDD configuration | ConfigSCell1 2 |  | TDDConf.1.1 |
|  | ConfigSCell1 3 |  | TDDConf.2.1 |
| BWchannel | ConfigSCell1 1,2 | MHz | 10: NRB,c = 52 |
|  | ConfigSCell1 3 |  | 40: NRB,c = 106 |
| Downlink initial BWP configuration |  |  | DLBWP.0.1 |
| Uplink initial BWP configuration |  |  | N/A |
| Downlink active non-dormant BWP-0 configuration |  |  | DLBWP.1.1 |
| Downlink active dormant BWP-1 configuration |  |  | DLBWP.1.1 |
| Uplink active BWP-0 configuration |  |  | N/A |
| TCI state |  |  | TCI.State.0 |
| CSI-RS configuration for CSI reporting, Non-dormant BWP | ConfigSCell1 1 |  | CSI-RS.1.1 FDD |
|  | ConfigSCell1 2 |  | CSI-RS.1.1 TDD |
|  | ConfigSCell1 3 |  | CSI-RS.2.1 TDD |
| CSI-RS configuration for CSI reporting, Dormant BWP | ConfigSCell1 1 |  | CSI-RS.1.6 FDD |
|  | ConfigSCell1 2 |  | CSI-RS.1.5 TDD |
|  | ConfigSCell1 3 |  | CSI-RS.2.6 TDD |
| TRS Configuration | ConfigSCell1 1 |  | TRS.1.1 FDD |
|  | ConfigSCell1 2 |  | TRS.1.1 TDD |
|  | ConfigSCell1 3 |  | TRS.1.2 TDD |
| PDSCH Reference measurement channel | ConfigSCell1 1 |  | SR.1.1 FDD |
|  | ConfigSCell1 2 |  | SR.1.1 TDD |
|  | ConfigSCell1 3 |  | SR.2.1 TDD |
| Dedicated CORESET parameters, Test 1,2 | ConfigSCell1 1 |  | CCR.1.1 FDD |
|  | ConfigSCell1 2 |  | CCR.1.1 TDD |
|  | ConfigSCell1 3 |  | CCR.2.1 TDD |
| Dedicated CORESET parameters, Test 3,4 | ConfigSCell1 1 |  | CCR.1.1 FDD |
|  | ConfigSCell1 2 |  | CCR.1.1 TDD |
|  | ConfigSCell1 3 |  | CCR.2.1 TDD |
| RMSI CORESET parameters | ConfigSCell1 1 |  | N/A |
|  | ConfigSCell1 2 |  |  |
|  | ConfigSCell1 3 |  |  |
| OCNG Pattern |  |  | OP.1 |
| SSB Configuration | ConfigSCell1 1,2 |  | SSB.1 FR1 |
|  | ConfigSCell1 3 |  | SSB.2 FR1 |
| SMTC configuration |  |  | SMTC.1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note1 |  |  |  |
| Noc Note2 | ConfigSCell1 1,2 | dBm/15kHz | -104 |
|  | ConfigSCell1 3 |  | -101 |
| Ês/Iot |  | dB | 17 |
| Ês/Noc |  | dB | 17 |
| SS-RSRP Note3 | ConfigSCell1 1,2 | dBm/SCS | -87 |
|  | ConfigSCell1 3 |  | -84 |
| Io Note3 | ConfigSCell1 1,2 | dBm/9.36 MHz | -59.0 |
|  | ConfigSCell1 3 | dBm/38.16 MHz | -52.9 |
| Propagation condition |  |  | AWGN |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |
| Note 1: OCNG shall be used such that the cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP, SCH_RP, and Io levels have been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |

Table A.6.5.6.4.2.1-5: NR Cell specific test parameters for NR SCell (Cell 3)

| Parameter |  | Unit | Cell 3 |
| --- | --- | --- | --- |
| Frequency range |  |  | FR1 |
| NR RF channel |  |  | 3 |
| Duplex mode | ConfigSCell2 1 |  | FDD |
|  | ConfigSCell2 2,3 |  | TDD |
| TDD configuration | ConfigSCell2 2 |  | TDDConf.1.1 |
|  | ConfigSCell2 3 |  | TDDConf.2.1 |
| BWchannel | ConfigSCell2 1,2 | MHz | 10: NRB,c = 52 |
|  | ConfigSCell2 3 |  | 40: NRB,c = 106 |
| Downlink initial BWP configuration |  |  | DLBWP.0.1 |
| Uplink initial BWP configuration |  |  | N/A |
| Downlink active non-dormant BWP-0 configuration |  |  | DLBWP.1.1 |
| Downlink active dormant BWP-1 configuration |  |  | DLBWP.1.1 |
| Uplink active BWP-0 configuration |  |  | N/A |
| TCI state |  |  | TCI.State.0 |
| CSI-RS configuration for CSI reporting, Non-dormant BWP | ConfigSCell2 1 |  | CSI-RS.1.1 FDD |
|  | ConfigSCell2 2 |  | CSI-RS.1.1 TDD |
|  | ConfigSCell2 3 |  | CSI-RS.2.1 TDD |
| CSI-RS configuration for CSI reporting, Dormant BWP | ConfigSCell2 1 |  | CSI-RS.1.6 FDD |
|  | ConfigSCell2 2 |  | CSI-RS.1.5 TDD |
|  | ConfigSCell2 3 |  | CSI-RS.2.6 TDD |
| TRS Configuration | ConfigSCell2 1 |  | TRS.1.1 FDD |
|  | ConfigSCell2 2 |  | TRS.1.1 TDD |
|  | ConfigSCell2 3 |  | TRS.1.2 TDD |
| PDSCH Reference measurement channel | ConfigSCell2 1 |  | SR.1.1 FDD |
|  | ConfigSCell2 2 |  | SR.1.1 TDD |
|  | ConfigSCell2 3 |  | SR.2.1 TDD |
| Dedicated CORESET parameters, Test 1,2 | ConfigSCell2 1 |  | CCR.1.1 FDD |
|  | ConfigSCell2 2 |  | CCR.1.1 TDD |
|  | ConfigSCell2 3 |  | CCR.2.1 TDD |
| Dedicated CORESET parameters, Test 3,4 | ConfigSCell2 1 |  | CCR.1.1 FDD |
|  | ConfigSCell2 2 |  | CCR.1.1 TDD |
|  | ConfigSCell2 3 |  | CCR.2.1 TDD |
| RMSI CORESET parameters | ConfigSCell2 1 |  | N/A |
|  | ConfigSCell2 2 |  |  |
|  | ConfigSCell2 3 |  |  |
| OCNG Pattern |  |  | OP.1 |
| SSB Configuration | ConfigSCell2 1,2 |  | SSB.1 FR1 |
|  | ConfigSCell2 3 |  | SSB.2 FR1 |
| SMTC configuration |  |  | SMTC.1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note1 |  |  |  |
| Noc Note2 | ConfigSCell2 1,2 | dBm/15kHz | -104 |
|  | ConfigSCell2 3 |  | -101 |
| Ês/Iot |  | dB | 17 |
| Ês/Noc |  | dB | 17 |
| SS-RSRP Note3 | ConfigSCell2 1,2 | dBm/SCS | -87 |
|  | ConfigSCell2 3 |  | -84 |
| Io Note3 | ConfigSCell2 1,2 | dBm/9.36 MHz | -59.0 |
|  | ConfigSCell2 3 | dBm/38.16 MHz | -52.9 |
| Propagation condition |  |  | AWGN |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |
| Note 1: OCNG shall be used such that the cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP, SCH_RP, and Io levels have been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |

###### A.6.5.6.4.2.2	Test Requirements

During T1, any interruption on PCell due to dormancy switching of SCells shall be within the requirement specified in clause 8.2.2.2.12.1.

During T2, interruptions on PCell due to CSI and RRM measurements on dormant SCells shall be within the interruption rate requirements specified in clauses 8.2.2.2.12.2 and 8.2.2.2.12.3, respectively.

During T3, any interruption on PCell due to dormancy switching of SCells shall be within the requirement specified in clause 8.2.2.2.12.1. Monitoring of PDCCH for SCell in SCell shall be resumed within the dormancy switching time specified in clause 8.6.2A.

For an event to be considered to be correct, all requirements above have to be fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs

##### A.6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA

A.6.5.6.5.1.1	Test Purpose and Environment

The purpose of this test is to verify requirements on the RRC-based DL BWP switch delay on multiple CCs defined in clause 8.6.

The supported test configurations for NR PCell are shown in Table A.6.5.6.5.1.1-1 below. The supported test configurations for NR SCell are shown in Table A.6.5.6.5.1.1-1A below. Test configuration for NR PCell and test configuration for NR SCell are chosen independently. The test scenario comprises of one NR PCell (Cell 1) and one NR SCell (Cell 2) as given in Table A.6.5.6.5.1.1-2. NR Cell-specific parameters are specified in Table A.6.5.6.5.1.1-3 and A.6.5.6.5.1.1-4 below.

PDCCHs indicating new transmissions shall be sent continuously on PCell (Cell 1) and SCell (Cell 2) to ensure that the UE would have ACK/NACK sending.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), Cell 2 (SCell) on radio channel 2 (SCC).

-	UE has bandwidth part BWP-1 in its RRC-configuration for PCell and SCell (Cell 2).

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 of initial condition in PCell and SCell (Cell 2).

All cells have constant signal levels throughout the test.

The test consists of 1 time period, with duration of T1.

During T1,

Time period T1 starts when a RRCReconfiguration with updated bandwidth part configuration for both PCell and SCell (Cell 2), sent from the test equipment to the UE, is completely received at the UE side in PCell’s and SCell’s slot # denoted i. The UE shall reconfigure its bandwidth part with the updated bandwidth part BWP-1 of final condition for both PCell and SCell (Cell 2).

The UE shall be able to receive PDSCH on PCell and SCell (Cell 2) from the first DL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TBWPswitchDelayRRC+DRRCNR slot length as defined in clause 8.6.3A.1 and starts to report valid ACK/NACK for the PCell and SCell (Cell 2) from the first UL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TBWPswitchDelayRRC+DRRCNR slot length+k1 on BWP-1 of final condition. The UE shall be continuously scheduled on PCell’s and SCell (Cell 2)’s BWP-1 of final condition starting from the first DL slot right after slot i+TRRCprocessingDelay+TBWPswitchDelayRRC+DRRCNR slot length.

TRRCprocessingDelay, TBWPswitchDelayRRC and DRRC are defined in clause 8.6.3A.1, N=2 in this test case.

The test equipment verifies the DL BWP switch time in PCell and SCell by counting the slots from the time when the RRC Reconfiguration message including updated BWP configuration is sent till the time when a vaild ACK/NACK is received.

Table A.6.5.6.5.1.1-1: DL BWP switch supported test configurations for NR PCell

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration |  |

Table A.6.5.6.5.1.1-1A: DL BWP switch supported test configurations for NR SCell

| ConfigSCell | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration |  |

Table A.6.5.6.5.1.1-2: General test parameters for DL BWP switch in SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1, 2 | Two NR radio channels are used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active SCell |  | Cell 2 | SCell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | For both PCell and SCell (Cell 2) |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cell on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cell on SCC. |
| Cell 2 timing offset to Cell 1 | s | 3 | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. |
| T1 | s | [0.2] |  |

Table A.6.5.6.5.1.1-3: NR Cell specific test parameters for NR PCell for DL BWP switch in SA

| Parameter |  |  |  | Unit | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  |  |  | FR1 |  |
| Duplex mode |  |  | Config 1 |  | FDD |  |
|  |  |  | Config 2,3 |  | TDD |  |
| TDD configuration |  |  | Config 1 |  | Not Applicable |  |
|  |  |  | Config 2 |  | TDDConf.1.1 |  |
|  |  |  | Config 3 |  | TDDConf.2.1 |  |
| BWchannel |  |  | Config 1,2 |  | 10 MHz: NRB,c = 52 |  |
|  |  |  | Config 3 |  | 40 MHz: NRB,c = 106 |  |
| Active BWP ID |  |  |  |  | 1 |  |
| Initial DL BWP Configuration |  |  |  |  | DLBWP.0.2Note4 |  |
| Initial UL BWP Configuration |  |  |  |  | ULBWP.0.2Note4 |  |
| Initial Condition | Active DL BWP-1 Configuration |  |  |  | DLBWP.1.3 Note4 |  |
|  | Active UL BWP-1 Configuration |  |  |  | ULBWP.1.3 Note4 |  |
| Final Condition | Active DL BWP-1 Configuration |  |  |  | DLBWP.1.1 Note4 |  |
|  | Active UL BWP-1 Configuration |  |  |  | ULBWP.1.1 Note4 |  |
| PDSCH Reference measurement channel |  |  | Config 1 |  | SR.1.1 FDD |  |
|  |  |  | Config 2 |  | SR.1.1 TDD |  |
|  |  |  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET parameters |  |  | Config 1 |  | CR.1.1 FDD |  |
|  |  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  |  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET parameters |  |  | Config 1 |  | CCR.1.1 FDD |  |
|  |  |  | Config 2 |  | CCR.1.1 TDD |  |
|  |  |  | Config 3 |  | CCR.2.1 TDD |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |
| SSB Configuration |  |  | Config 1,2 |  | SSB.1 FR1 |  |
|  |  |  | Config 3 |  | SSB.2 FR1 |  |
| SMTC Configuration |  |  |  |  | SMTC.1 |  |
| TRS Configuration |  |  | Config 1 |  | TRS.1.1 FDD |  |
|  |  |  | Config 2 |  | TRS.1.1 TDD |  |
|  |  |  | Config 3 |  | TRS.1.2 TDD |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 1x2 Low |  |
| Propagation Condition |  |  |  |  | AWGN |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| NocNote 2 |  | Config 1,2 |  | dBm/SCS | -104 |
|  |  | Config 3 |  |  | -101 |
| SS-RSRP Note 3 |  | Config 1,2 |  | dBm/SCS | -87 |
|  |  | Config 3 |  |  | -84 |
| Ês/Iot |  |  |  | dB | 17 |
| Ês/Noc |  |  |  | dB | 17 |
| IoNote3 |  |  | Config 1,2 | dBm/ 9.36MHz | -58.96 |
|  |  |  | Config 3 | dBm/ 38.16MHz | -52.86 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3 SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. |  |  |  |  |  |

Table A.6.5.6.5.1.1-4: NR Cell specific test parameters for NR SCell for DL BWP switch in SA

| Parameter |  |  |  | Unit | Cell 2 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  |  |  | FR1 |
| Duplex mode |  |  | ConfigSCell 1 |  | FDD |
|  |  |  | ConfigSCell 2,3 |  | TDD |
| TDD configuration |  |  | ConfigSCell 1 |  | Not Applicable |
|  |  |  | ConfigSCell 2 |  | TDDConf.1.1 |
|  |  |  | ConfigSCell 3 |  | TDDConf.2.1 |
| BWchannel |  |  | ConfigSCell 1,2 |  | 10 MHz: NRB,c = 52 |
|  |  |  | ConfigSCell 3 |  | 40 MHz: NRB,c = 106 |
| Active BWP ID |  |  |  |  | 1 |
| Initial DL BWP Configuration |  |  |  |  | DLBWP.0.2Note4 |
| Initial UL BWP Configuration |  |  |  |  | ULBWP.0.2Note4 |
| Initial Condition | Active DL BWP-1 Configuration |  |  |  | DLBWP.1.3 Note4 |
|  | Active UL BWP-1 Configuration |  |  |  | ULBWP.1.3 Note4 |
| Final Condition | Active DL BWP-1 Configuration |  |  |  | DLBWP.1.1 Note4 |
|  | Active UL BWP-1 Configuration |  |  |  | ULBWP.1.1 Note4 |
| PDSCH Reference measurement channel |  |  | ConfigSCell 1 |  | SR.1.1 FDD |
|  |  |  | ConfigSCell 2 |  | SR.1.1 TDD |
|  |  |  | ConfigSCell 3 |  | SR.2.1 TDD |
| RMSI CORESET parameters |  |  | ConfigSCell 1 |  | CR.1.1 FDD |
|  |  |  | ConfigSCell 2 |  | CR.1.1 TDD |
|  |  |  | ConfigSCell 3 |  | CR.2.1 TDD |
| Dedicated CORESET parameters |  |  | ConfigSCell 1 |  | CCR.1.1 FDD |
|  |  |  | ConfigSCell 2 |  | CCR.1.1 TDD |
|  |  |  | ConfigSCell 3 |  | CCR.2.1 TDD |
| OCNG Patterns |  |  |  |  | OP.1 |
| SSB Configuration |  |  | ConfigSCell 1,2 |  | SSB.1 FR1 |
|  |  |  | ConfigSCell 3 |  | SSB.2 FR1 |
| SMTC Configuration |  |  |  |  | SMTC.1 |
| TRS Configuration |  |  | ConfigSCell 1 |  | TRS.1.1 FDD |
|  |  |  | ConfigSCell 2 |  | TRS.1.1 TDD |
|  |  |  | ConfigSCell 3 |  | TRS.1.2 TDD |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 1x2 Low |
| Propagation Condition |  |  |  |  | AWGN |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| NocNote 2 |  | ConfigSCell 1,2 |  | dBm/SCS | -104 |
|  |  | ConfigSCell 3 |  |  | -101 |
| SS-RSRP Note 3 |  | ConfigSCell 1,2 |  | dBm/SCS | -87 |
|  |  | ConfigSCell 3 |  |  | -84 |
| Ês/Iot |  |  |  | dB | 17 |
| Ês/Noc |  |  |  | dB | 17 |
| IoNote3 |  |  | ConfigSCell 1,2 | dBm/ 9.36MHz | -58.96 |
|  |  |  | ConfigSCell 3 | dBm/ 38.16MHz | -52.86 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3 SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. |  |  |  |  |  |

A.6.5.6.5.1.2	Test Requirements

During T1, the UE shall be ready for the reception of uplink grant for both PCell and SCell (Cell 2) from the first DL slot that occurs right after the begining of slot (i+TRRCprocessingDelay+TBWPswitchDelayRRC+DRRCNR slot length) and starts to report valid ACK/NACK for both PCell and SCell (Cell 2) from the first UL slot that occurs after the beginning of DL slot (i+ TRRCprocessingDelay+TBWPswitchDelayRRC+DRRCNR slot length+k1).

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [7].

Depending on UE capability bwp-SwitchingDelay [2], UE shall finish BWP switch on PCell and SCell (Cell 2) within the time duration TRRCprocessingDelay+TBWPswitchDelayRRC+DRRC defined in 8.6.3A.1.

All of the above test requirements shall be fulfilled in order for the observed PCell and SCell (Cell 2) active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	During T1 if there are no uplink resources for reporting the ACK/NACK in the first UL slot that occurs after beginning of DL slot (i+ TRRCprocessingDelay+TBWPswitchDelayRRC+DRRCNR slot length +k1), then the UE shall use the next available uplink resource for reporting the corresponding ACK/NACK.

### A.6.5.7	DL interruptions at switching between two uplink carriers

#### A.6.5.7.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA

##### A.6.5.7.1.1	Test Purpose and Environment

The purpose of this test is to verify DL interruption requirements during UE dynamic switching between two uplink carriers defined in clause 8.2.2.2.10. The test case is applicable for an uplink band pair of an inter-band FDD-TDD CA configuration when the capability uplinkTxSwitchingPeriod is present.

There are two cells: FR1 FDD PCell (Cell 1), FR1 TDD SCell (Cell 2). The test parameters for the two cells are given in Table A.6.5.7.1.1-1, Table A.6.5.7.1.1-2 and Table A.6.5.7.1.1-3 below.

For NR FDD carrier (Cell 1), aperiodic CSI-RS for L1-RSRP reporting is triggered with power boosting 6dB on the following symbol in the slot overlapping with the special slot of the NR TDD carrier (Cell 2):

symbol#12 if UE does not report uplinkTxSwitching-DL-Interruption-r16;

otherwise,

symbol #8 if UE capability uplinkTxSwitchingPeriod is 210us or

symbol #9 if UE capability uplinkTxSwitchingPeriod is 140us or

symbol #10 if UE capability uplinkTxSwitchingPeriod is 35us.

For NR TDD carrier (Cell 2), aperiodic CSI-RS for L1-RSRP reporting is configured with power boosting 6dB on the following symbol in the special slot:

symbol#10 if UE does not report uplinkTxSwitching-DL-Interruption-r16;

otherwise,

symbol #4 if UE capability uplinkTxSwitchingPeriod is 210us or

symbol #5 if UE capability uplinkTxSwitchingPeriod is 140us or

symbol #8 if UE capability uplinkTxSwitchingPeriod is 35us.

This test verifies that the UE correctly report the L1-RSRP reporting. The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, uplinkTxSwitching is indicated to UE.

Table A.6.5.7.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | NR Cell 1: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode NR Cell 2: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.6.5.7.1.1-2: General test parameters for DL interruptions at switching between two uplink carriers in FDD-TDD CA

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | Config 1 | 1, 2 | Two radio channels are used for this test. |
| Active cell |  | Config 1 | Cell 1: FR1 PCell Cell 2: FR1 SCell | FR1 PCell on RF channel number 1 FR1 SCell on RF channel number 2 |
| CP length |  | Config 1 | Normal |  |
| DRX |  | Config 1 | OFF |  |
| Measurement gap pattern Id |  | Config 1 | OFF |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| CSI-RS configuration for L1-RSRP reporting |  | Config 1 | Cell 1: CSI-RS.1.5 FDD Cell 2: CSI-RS.2.5 TDD |  |
| T1 | s | Config 1 | 5 |  |

Table A.6.5.7.1.1-3: Cell specific test parameters for DL interruptions at switching between two uplink carriers in FDD-TDD CA

| Parameter |  | Unit | Cell1 | Cell2 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR1 | FR1 |
| Duplex mode | Config 1 |  | FDD | TDD |
| TDD configuration | Config 1 |  | N/A | TDDConf.2.1 except that: S=’11DL: 1GP:2UL’; nrofDownlinkSymbols: 11 nrofUplinkSymbols: 2 |
| BWchannel | Config 1 |  | 10 MHz: NRB,c = 52 | 40 MHz: NRB,c = 106 |
| Initial BWP Configuration | Config 1 |  | DLBWP.0.1 | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1 |  | DLBWP.1.1 | DLBWP.1.1 |
| UL dedicated BWP configuration | Config 1 |  | ULBWP.1.1 | ULBWP.1.1 |
| SRS configuration | Config 1 |  | SRS configuration in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0resourceMappingnrofSymbols: n2 | SRS configuration in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD | SR.2.1 TDD |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD | CR.2.1 TDD |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD | CCR.2.1 TDD |
| OCNG Patterns |  |  | OP.1 | OP.1 |
| SMTC Configuration |  |  | SMTC.1 | SMTC.1 |
| SSB Configuration | Config 1 |  | SSB.1 FR1 | SSB.2 FR1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 2x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| NocNote 2 |  | dBm/15 kHz | -104 | -104 |
| SS-RSRP Note 3 |  | dBm/SSB SCS | -87 | -84 |
| CSI-RS RSRP Note6 |  | dBm/SCS | -81 | -78 |
| Ês/Iot |  | dB | 17 | 17 |
| Ês/Noc |  | dB | 17 | 17 |
| NocNote 2 | Config 1 | dBm/SCS | -104 | -101 |
| IoNote3 on symbols without CSI-RS | Config 1 | dBm/9.36 MHz | -58.96 | - |
|  |  | dBm/ 38.16MHz | - | -52.86 |
| IoNote6 on symbols with CSI-RS | Config 1 | dBm/9.36 MHz | -56.58 | - |
|  |  | dBm/ 38.16MHz | - | -50.5 |
| Time offset to Cell1 Note 5 |  | s | - | 0 |
| Propagation Condition |  |  | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 6: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.6.5.7.1.2	Test Requirements

The UE behaviour follows the requirements defined in clause 8.2.2.2.10.

UE shall send L1-RSRP report while meeting the accuracy requirements defined in clause 10.1.19.2.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.7.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA

##### A.6.5.7.2.1	Test Purpose and Environment

The purpose of this test is to verify DL interruption requirements during UE dynamic switching between two uplink carriers defined in clause 8.2.2.2.10. The test case is applicable for an uplink band pair of an inter-band TDD-TDD CA configuration when the capability uplinkTxSwitchingPeriod is present.

There are two cells: FR1 TDD PCell (Cell 1), FR1 TDD SCell (Cell 2). The test parameters for the two cells are given in Table A.6.5.7.2.1-1, Table A.6.5.7.2.1-2 and Table A.6.5.7.2.1-3 below.

For NR TDD PCell (Cell 1), aperiodic CSI-RS for L1-RSRP reporting is triggered with power boosting [6dB] on the following symbol in the special

slot:symbol#10 if UE does not report uplinkTxSwitching-DL-Interruption-r16;

otherwise,

symbol #4 if UE capability uplinkTxSwitchingPeriod is 210us or

symbol #5 if UE capability uplinkTxSwitchingPeriod is 140us or

symbol #8 if UE capability uplinkTxSwitchingPeriod is 35us.

For NR TDD SCell (Cell 2), aperiodic CSI-RS for L1-RSRP reporting is configured with power boosting [6dB] on the following symbol on the 2nd special slot of every 8 slots:

symbol#10 if UE does not report uplinkTxSwitching-DL-Interruption-r16;

otherwise,

symbol #4 if UE capability uplinkTxSwitchingPeriod is 210us or

symbol #5 if UE capability uplinkTxSwitchingPeriod is 140us or

symbol #8 if UE capability uplinkTxSwitchingPeriod is 35us.

This test verifies that the UE correctly report the L1-RSRP reporting. The test case is only applicable to UE which supports simultaneousRxTxInterBandCA.

The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, uplinkTxSwitching is indicated to UE.

Table A.6.5.7.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | NR Cell 1: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR Cell 2: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.6.5.7.2.1-2: General test parameters for DL interruptions at switching between two uplink carriers in TDD-TDD CA

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | Config 1 | 1, 2 | Two radio channels are used for this test. |
| Active cell |  | Config 1 | Cell 1: FR1 PCell Cell 2: FR1 SCell | FR1 PCell on RF channel number 1 FR1 SCell on RF channel number 2 |
| CP length |  | Config 1 | Normal |  |
| DRX |  | Config 1 | OFF |  |
| Measurement gap pattern Id |  | Config 1 | OFF |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| CSI-RS configuration for L1-RSRP reporting |  | Config 1 | Cell 1: CSI-RS.2.5 TDD Cell 2: CSI-RS.2.5 TDD |  |
| T1 | s | Config 1 | 5 |  |

Table A.6.5.7.2.1-3: Cell specific test parameters for DL interruptions at switching between two uplink carriers in TDD-TDD CA

| Parameter |  | Unit | Cell1 | Cell2 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR1 | FR1 |
| Duplex mode | Config 1 |  | TDD | TDD |
| TDD configuration | Config 1 |  | TDDConf.2.1 except that S=’1 1DL: :2UL’; nrofDownlinkSymbols: 11 nrofUplinkSymbols: 2 | TDDConf.2.2 |
| BWchannel | Config 1 |  | 40 MHz: NRB,c = 106 | 40 MHz: NRB,c = 106 |
| Initial BWP Configuration | Config 1 |  | DLBWP.0.1 | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1 |  | DLBWP.1.1 | DLBWP.1.1 |
| UL dedicated BWP configuration | Config 1 |  | ULBWP.1.1 | ULBWP.1.1 |
| SRS configuration | Config 1 |  | SRS configuration in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 | SRS configuration in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 |
| PDSCH Reference measurement channel | Config 1 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET parameters | Config 1 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET parameters | Config 1 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| OCNG Patterns |  |  | OP.1 | OP.1 |
| SMTC Configuration |  |  | SMTC.1 | SMTC.1 |
| SSB Configuration | Config 1 |  | SSB.2 FR1 | SSB.2 FR1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 2x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| NocNote 2 |  | dBm/15 kHz | -104 | -104 |
| SS-RSRP Note 3 |  | dBm/SSB SCS | 84 | -84 |
| CSI-RS RSRP Note6 |  | dBm/SCS | -78 | -78 |
| Ês/Iot |  | dB | 17 | 17 |
| Ês/Noc |  | dB | 17 | 17 |
| NocNote 2 | Config 1 | dBm/SCS | -104 | -101 |
| IoNote3 on symbols without CSI-RS | Config 1 | dBm/ 38.16MHz | -52.86 | -52.86 |
| IoNote6 on symbols with CSI-RS | Config 1 | dBm/ 38.16MHz | -50.5 | -50.5 |
| Time offset to Cell1 Note 5 |  | s | - | 0 |
| Propagation Condition |  |  | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 6: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.6.5.7.2.2	Test Requirements

The UE behaviour follows the requirements defined in clause 8.2.2.2.10.

UE shall send L1-RSRP report while meeting the accuracy requirements defined in clause 10.1.19.2.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.6.5.7A	DL interruptions at switching between two uplink carriers with two transmit antenna connectors

#### A.6.5.7A.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA

##### A.6.5.7A.1.1	Test Purpose and Environment

The purpose of this test is to verify DL interruption requirements during UE dynamic switching between two uplink carriers defined in clause 8.2.2.2.10A. The test case is applicable for an uplink band pair of an inter-band FDD-TDD CA configuration when the capability uplinkTxSwitchingPeriod2T2T is present, where NR UL carrier 1 is capable of two transmit antenna connectors and NR UL carrier 2 is capable of two transmit antenna connectors, and the two uplink carriers are in different bands with different carrier frequencies.

There are two cells: FR1 FDD PCell (Cell 1), FR1 TDD SCell (Cell 2). The test parameters for the two cells are given in Table A.6.5.7A.1.1-1, Table A.6.5.7A.1.1-2 and Table A.6.5.7A.1.1-3 below.

For NR FDD carrier (Cell 1), aperiodic CSI-RS for L1-RSRP reporting is triggered with power boosting [6dB] on the following symbol in the slot overlapping with the special slot of the NR TDD carrier (Cell 2):

-	symbol#12 if UE does not report uplinkTxSwitching-DL-Interruption-r16;

-	otherwise,

-	symbol #8 if UE capability uplinkTxSwitchingPeriod2T2T is 210us or

-	symbol #9 if UE capability uplinkTxSwitchingPeriod2T2T is 140us or

-	symbol #10 if UE capability uplinkTxSwitchingPeriod2T2T is 35us.

For NR TDD carrier (Cell 2), aperiodic CSI-RS for L1-RSRP reporting is configured with power boosting [6dB] on the following symbol in the special slot:

-	symbol#10 if UE does not report uplinkTxSwitching-DL-Interruption-r16;

-	otherwise,

-	symbol #4 if UE capability uplinkTxSwitchingPeriod2T2T is 210us or

-	symbol #5 if UE capability uplinkTxSwitchingPeriod2T2T is 140us or

-	symbol #8 if UE capability uplinkTxSwitchingPeriod2T2T is 35us.

This test verifies that the UE correctly report the L1-RSRP reporting. The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, uplinkTxSwitching is indicated to UE.

Table A.6.5.7A.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | NR Cell 1: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode NR Cell 2: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.6.5.7A.1.1-2: General test parameters for DL interruptions at switching between two uplink carriers in FDD-TDD CA

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | Config 1 | 1, 2 | Two radio channels are used for this test. |
| Active cell |  | Config 1 | Cell 1: FR1 PCell Cell 2: FR1 SCell | FR1 PCell on RF channel number 1 FR1 SCell on RF channel number 2 |
| CP length |  | Config 1 | Normal |  |
| DRX |  | Config 1 | OFF |  |
| Measurement gap pattern Id |  | Config 1 | OFF |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| CSI-RS configuration for L1-RSRP reporting |  | Config 1 | Cell 1: CSI-RS.1.5 FDD Cell 2: CSI-RS.2.5 TDD |  |
| T1 | s | Config 1 | 5 |  |

Table A.6.5.7A.1.1-3: Cell specific test parameters for DL interruptions at switching between two uplink carriers in FDD-TDD CA

| Parameter |  | Unit | Cell1 | Cell2 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR1 | FR1 |
| Duplex mode | Config 1 |  | FDD | TDD |
| TDD configuration | Config 1 |  | N/A | TDDConf.2.1 except that: S=’11DL: 1GP:2UL’; nrofDownlinkSymbols: 11 nrofUplinkSymbols: 2 |
| BWchannel | Config 1 |  | 10 MHz: NRB,c = 52 | 40 MHz: NRB,c = 106 |
| Initial BWP Configuration | Config 1 |  | DLBWP.0.1 | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1 |  | DLBWP.1.1 | DLBWP.1.1 |
| UL dedicated BWP configuration | Config 1 |  | ULBWP.1.1 | ULBWP.1.1 |
| SRS configuration | Config 1 |  | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD | SR.2.1 TDD |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD | CR.2.1 TDD |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD | CCR.2.1 TDD |
| OCNG Patterns |  |  | OP.1 | OP.1 |
| SMTC Configuration |  |  | SMTC.1 | SMTC.1 |
| SSB Configuration | Config 1 |  | SSB.1 FR1 | SSB.2 FR1 |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low | 2x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| NocNote 2 |  | dBm/15 kHz | -104 | -104 |
| SS-RSRP Note 3 |  | dBm/SCS | -87 | -84 |
| Ês/Iot |  | dB | 17 | 17 |
| Ês/Noc |  | dB | 17 | 17 |
| NocNote 2 | Config 1 | dBm/SCS | -104 | -101 |
| IoNote3 | Config 1 | dBm/9.36 MHz | -58.96 | - |
|  |  | dBm/ 38.16MHz | - | -52.86 |
| Time offset to Cell1 Note 5 |  | s | - | 0 |
| Propagation Condition |  |  | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |

##### A.6.5.7A.1.2	Test Requirements

The UE behaviour follows the requirements defined in clause 8.2.2.2.10A.

UE shall send L1-RSRP report while meeting the accuracy requirements defined in clause 10.1.19.2.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.7A.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA

##### A.6.5.7A.2.1	Test Purpose and Environment

The purpose of this test is to verify DL interruption requirements during UE dynamic switching between two uplink carriers defined in clause 8.2.2.2.10A. The test case is applicable for an uplink band pair of an inter-band TDD-TDD CA configuration when the capability uplinkTxSwitchingPeriod2T2T is present, where NR UL carrier 1 is capable of two transmit antenna connectors and NR UL carrier 2 is capable of two transmit antenna connectors, and the two uplink carriers are in different bands with different carrier frequencies.

There are two cells: FR1 TDD PCell (Cell 1), FR1 TDD SCell (Cell 2). The test parameters for the two cells are given in Table A.6.5.7A.2.1-1, Table A.6.5.7A.2.1-2 and Table A.6.5.7A.2.1-3 below.

For NR TDD PCell (Cell 1), aperiodic CSI-RS for L1-RSRP reporting is triggered with power boosting [6dB] on the following symbol in the special slot:

-	symbol#10 if UE does not report uplinkTxSwitching-DL-Interruption-r16;

-	otherwise,

-	symbol #4 if UE capability uplinkTxSwitchingPeriod2T2T is 210us or

-s	ymbol #5 if UE capability uplinkTxSwitchingPeriod2T2T is 140us or

-	symbol #8 if UE capability uplinkTxSwitchingPeriod2T2T is 35us.

For NR TDD SCell (Cell 2), aperiodic CSI-RS for L1-RSRP reporting is configured with power boosting [6dB] on the following symbol on the 2nd special slot of every 8 slots:

-	symbol#10 if UE does not report uplinkTxSwitching-DL-Interruption-r16;

-	otherwise,

-	symbol #4 if UE capability uplinkTxSwitchingPeriod2T2T is 210us or

-	symbol #5 if UE capability uplinkTxSwitchingPeriod2T2T is 140us or

-	symbol #8 if UE capability uplinkTxSwitchingPeriod2T2T is 35us.

This test verifies that the UE correctly report the L1-RSRP reporting. The test case is only applicable to UE which supports simultaneousRxTxInterBandCA.

The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, uplinkTxSwitching is indicated to UE.

Table A.6.5.7A.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | NR Cell 1: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR Cell 2: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.6.5.7A.2.1-2: General test parameters for DL interruptions at switching between two uplink carriers in TDD-TDD CA

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | Config 1 | 1, 2 | Two radio channels are used for this test. |
| Active cell |  | Config 1 | Cell 1: FR1 PCell Cell 2: FR1 SCell | FR1 PCell on RF channel number 1 FR1 SCell on RF channel number 2 |
| CP length |  | Config 1 | Normal |  |
| DRX |  | Config 1 | OFF |  |
| Measurement gap pattern Id |  | Config 1 | OFF |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| CSI-RS configuration for L1-RSRP reporting |  | Config 1 | Cell 1: CSI-RS.2.5 TDD Cell 2: CSI-RS.2.5 TDD |  |
| T1 | s | Config 1 | 5 |  |

Table A.6.5.7A.2.1-3: Cell specific test parameters for DL interruptions at switching between two uplink carriers in TDD-TDD CA

| Parameter |  | Unit | Cell1 | Cell2 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR1 | FR1 |
| Duplex mode | Config 1 |  | TDD | TDD |
| TDD configuration | Config 1 |  | TDDConf.2.1 except that S=’1 1DL: :2UL’; nrofDownlinkSymbols: 11 nrofUplinkSymbols: 2 | TDDConf.2.2 |
| BWchannel | Config 1 |  | 40 MHz: NRB,c = 106 | 40 MHz: NRB,c = 106 |
| Initial BWP Configuration | Config 1 |  | DLBWP.0.1 | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1 |  | DLBWP.1.1 | DLBWP.1.1 |
| UL dedicated BWP configuration | Config 1 |  | ULBWP.1.1 | ULBWP.1.1 |
| SRS configuration | Config 1 |  | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 |
| PDSCH Reference measurement channel | Config 1 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET parameters | Config 1 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET parameters | Config 1 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| OCNG Patterns |  |  | OP.1 | OP.1 |
| SMTC Configuration |  |  | SMTC.1 | SMTC.1 |
| SSB Configuration | Config 1 |  | SSB.2 FR1 | SSB.2 FR1 |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low | 2x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| NocNote 2 |  | dBm/15 kHz | -104 | -104 |
| SS-RSRP Note 3 |  | dBm/SCS | -84 | -84 |
| Ês/Iot |  | dB | 17 | 17 |
| Ês/Noc |  | dB | 17 | 17 |
| NocNote 2 | Config 1 | dBm/SCS | -104 | -101 |
| IoNote3 | Config 1 | dBm/9.36 MHz | -58.96 | - |
|  |  | dBm/ 38.16MHz | - | -52.86 |
| Time offset to Cell1 Note 5 |  | s | - | 0 |
| Propagation Condition |  |  | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |

##### A.6.5.7A.2.2	Test Requirements

The UE behaviour follows the requirements defined in clause 8.2.2.2.10A.

UE shall send L1-RSRP report while meeting the accuracy requirements defined in clause 10.1.19.2.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.6.5.7B	DL interruptions at switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors

#### A.6.5.7B.1	DL interruptions at switching between two uplink bands in FDD-TDD CA

##### A.6.5.7B.1.1	Test Purpose and Environment

The purpose of this test is to verify DL interruption requirements during UE dynamic switching between two uplink bands defined in clause 8.2.2.2.10B. The test case is applicable for an uplink band pair of an inter-band UL CA configuration when the capability uplinkTxSwitchingPeriod is present, where NR UL carrier 1 in band A is capable of one transmit antenna connector, NR UL carrier 2 and carrier 3 in band B are capable of two transmit antenna connectors. NR UL carrier 2 and carrier 3 are two contiguous aggregated carriers, and band A and band B are different bands with different carrier frequencies.

There are three cells: FR1 FDD PCell (Cell 1), FR1 TDD SCell (Cell 2) and FR1 TDD SCell (Cell 3) where cell 1 in band A is with 1TX, cell2 and cell 3 in band B with 2Tx, cell2 and cell3 are two contiguous aggregated carriers. The test parameters for the three cells are given in Table A.6.5.7B.1.1-1, Table A.6.5.7B.1.1-2 and Table A.6.5.7B.1.1-3 below.

For NR FDD carrier (Cell 1), aperiodic CSI-RS for L1-RSRP reporting is triggered with power boosting [6dB] on the following symbol in the slot overlapping with the special slot of the NR TDD carrier (Cell 2):

-	symbol#12 if UE does not report uplinkTxSwitching-DL-Interruption-r16;

-	otherwise,

-	symbol #8 if UE capability uplinkTxSwitchingPeriod is 210us or

-	symbol #9 if UE capability uplinkTxSwitchingPeriod is 140us or

-	symbol #10 if UE capability uplinkTxSwitchingPeriod is 35us.

For NR TDD Cell 2 and NR TDD Cell 3, aperiodic CSI-RS for L1-RSRP reporting is configured with power boosting [6dB] on the following symbol in the special slot:

-	symbol#10 if UE does not report uplinkTxSwitching-DL-Interruption-r16;

-	otherwise,

-	symbol #4 if UE capability uplinkTxSwitchingPeriod is 210us or

-	symbol #5 if UE capability uplinkTxSwitchingPeriod is 140us or

-	symbol #8 if UE capability uplinkTxSwitchingPeriod is 35us.

This test verifies that the UE correctly report the L1-RSRP reporting. The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, uplinkTxSwitching is indicated to UE.

Table A.6.5.7B.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | NR Cell 1: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode NR Cell 2: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR Cell 3: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.6.5.7B.1.1-2: General test parameters for DL interruptions at switching between two uplink bands in FDD-TDD CA

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | Config 1 | 1, 2, 3 | Three radio channels are used for this test. |
| Active cell |  | Config 1 | Cell 1: FR1 PCell Cell 2: FR1 SCell Cell 3: FR1 SCell | Cell1: FR1 PCell on RF channel number 1 in band A Cell 2: FR1 SCell on RF channel number 2 in band B Cell 3: FR1 SCell on RF channel number 3 in band B Note: Cell 2 and Cell 3 are two contiguous aggregated carriers |
| CP length |  | Config 1 | Normal |  |
| DRX |  | Config 1 | OFF |  |
| Measurement gap pattern Id |  | Config 1 | OFF |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| CSI-RS configuration for L1-RSRP reporting |  | Config 1 | Cell 1: CSI-RS.1.5 FDD Cell 2: CSI-RS.2.5 TDD Cell 3: CSI-RS.2.5 TDD |  |
| T1 | s | Config 1 | 5 |  |

Table A.6.5.7B.1.1-3: Cell specific test parameters for DL interruptions at switching between two uplink bands in FDD-TDD CA

| Parameter |  |  | Unit | Cell1 | Cell2 | Cell3 |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  |  | FR1 | FR1 | FR1 |
| Duplex mode |  | Config 1 |  | FDD | TDD | TDD |
| TDD configuration |  | Config 1 |  | N/A | TDDConf.2.1 except that: S=’11DL: 1GP:2UL’; nrofDownlinkSymbols: 11 nrofUplinkSymbols: 2 | TDDConf.2.1 except that: S=’11DL: 1GP:2UL’; nrofDownlinkSymbols: 11 nrofUplinkSymbols: 2 |
| BWchannel |  | Config 1 |  | 10 MHz: NRB,c = 52 | 40 MHz: NRB,c = 106 | 40 MHz: NRB,c = 106 |
| Initial BWP Configuration |  | Config 1 |  | DLBWP.0.1 | DLBWP.0.1 | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1 |  | DLBWP.1.1 | DLBWP.1.1 | DLBWP.1.1 |
| UL dedicated BWP configuration |  | Config 1 |  | ULBWP.1.1 | ULBWP.1.1 | ULBWP.1.1 |
| SRS configuration |  | Config 1 |  | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0resourceMappingnrofSymbols: n2 | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET parameters |  | Config 1 |  | CR.1.1 FDD | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET parameters |  | Config 1 |  | CCR.1.1 FDD | CCR.2.1 TDD | CCR.2.1 TDD |
| OCNG Patterns |  |  |  | OP.1 | OP.1 | OP.1 |
| SMTC Configuration |  |  |  | SMTC.1 | SMTC.1 | SMTC.1 |
| SSB Configuration | Config 1 |  |  | SSB.1 FR1 | SSB.2 FR1 | SSB.2 FR1 |
| Correlation Matrix and Antenna Configuration |  |  |  | 1x2 Low | 2x2 Low | 2x2 Low |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| NocNote 2 |  |  | dBm/15 kHz | -104 | -104 | -104 |
| SS-RSRP Note 3 |  |  | dBm/SCS | -87 | -87 | -87 |
| Ês/Iot |  |  | dB | 17 | 14 | 14 |
| Ês/Noc |  |  | dB | 17 | 14 | 14 |
| NocNote 2 | Config 1 |  | dBm/SCS | -104 | -101 | -101 |
| IoNote3 | Config 1 |  | dBm/9.36 MHz | -58.96 | - | - |
|  |  |  | dBm/ 38.16MHz | - | -55.79 | -55.79 |
| Time offset to Cell1 Note 5 |  |  | s | - | 0 | 0 |
| Propagation Condition |  |  |  | AWGN | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |  |  |

##### A.6.5.7B.1.2	Test Requirements

The UE behaviour follows the requirements defined in clause 8.2.2.2.10B.

UE shall send L1-RSRP report while meeting the accuracy requirements defined in clause 10.1.19.2.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.7B.2	DL interruptions at switching between two uplink bands in TDD-TDD CA

##### A.6.5.7B.2.1	Test Purpose and Environment

The purpose of this test is to verify DL interruption requirements during UE dynamic switching between two uplink bands defined in clause 8.2.2.2.10B. The test case is applicable for an uplink band pair of an inter-band UL CA configuration when the capability uplinkTxSwitchingPeriod is present, where NR UL carrier 1 in band A is capable of one transmit antenna connector, NR UL carrier 2 and carrier 3 in band B are capable of two transmit antenna connectors. NR UL carrier 2 and carrier 3 are two contiguous aggregated carriers, and band A and band B are different bands with different carrier frequencies.

There are three cells: FR1 TDD PCell (Cell 1), FR1 TDD SCell (Cell 2) and FR1 TDD SCell (Cell 3) where cell 1 in band A is with 1TX, cell2 and cell 3 in band B with 2Tx, cell2 and cell3 are two contiguous aggregated carriers.The test parameters for the three cells are given in Table A.6.5.7B.2.1-1, Table A.6.5.7B.2.1-2 and Table A.6.5.7B.2.1-3 below.

For NR TDD PCell (Cell 1), aperiodic CSI-RS for L1-RSRP reporting is triggered with power boosting [6dB] on the following symbol in the special slot:

-	symbol#10 if UE does not report uplinkTxSwitching-DL-Interruption-r16;

-	otherwise,

-	symbol #4 if UE capability uplinkTxSwitchingPeriod is 210us or

-	symbol #5 if UE capability uplinkTxSwitchingPeriod is 140us or

-	symbol #8 if UE capability uplinkTxSwitchingPeriod is 35us.

For NR TDD Cell 2 and NR TDD Cell 3, aperiodic CSI-RS for L1-RSRP reporting is configured with power boosting [6dB] on the following symbol on the 2nd special slot of every 8 slots:

-	symbol#10 if UE does not report uplinkTxSwitching-DL-Interruption-r16;

-	otherwise,

-	symbol #4 if UE capability uplinkTxSwitchingPeriod is 210us or

-	symbol #5 if UE capability uplinkTxSwitchingPeriod is 140us or

-	symbol #8 if UE capability uplinkTxSwitchingPeriod is 35us.

This test verifies that the UE correctly report the L1-RSRP reporting. The test case is only applicable to UE which supports simultaneousRxTxInterBandCA.

The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, uplinkTxSwitching is indicated to UE.

Table A.6.5.7B.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | NR Cell 1: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR Cell 2: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR Cell 3: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.6.5.7B.2.1-2: General test parameters for DL interruptions at switching between two uplink bands in TDD-TDD CA

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | Config 1 | 1, 2, 3 | Two radio channels are used for this test. |
| Active cell |  | Config 1 | Cell 1: FR1 PCell Cell 2: FR1 SCell Cell 3: FR1 SCell | Cell1: FR1 PCell on RF channel number 1 in band A Cell 2: FR1 SCell on RF channel number 2 in band B Cell 3: FR1 SCell on RF channel number 3 in band B Note: Cell 2 and Cell 3 are two contiguous aggregated carriers |
| CP length |  | Config 1 | Normal |  |
| DRX |  | Config 1 | OFF |  |
| Measurement gap pattern Id |  | Config 1 | OFF |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| CSI-RS configuration for L1-RSRP reporting |  | Config 1 | Cell 1: CSI-RS.2.5 TDD Cell 2: CSI-RS.2.5 TDD Cell 3: CSI-RS.2.5 TDD |  |
| T1 | s | Config 1 | 5 |  |

Table A.6.5.7B.2.1-3: Cell specific test parameters for DL interruptions at switching between two uplink bands in TDD-TDD CA

| Parameter |  | Unit | Cell1 | Cell2 | Cell3 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR1 | FR1 | FR1 |
| Duplex mode | Config 1 |  | TDD | TDD | TDD |
| TDD configuration | Config 1 |  | TDDConf.2.1 except that S=’1 1DL: :2UL’; nrofDownlinkSymbols: 11 nrofUplinkSymbols: 2 | TDDConf.2.2 | TDDConf.2.2 |
| BWchannel | Config 1 |  | 40 MHz: NRB,c = 106 | 40 MHz: NRB,c = 106 | 40 MHz: NRB,c = 106 |
| Initial BWP Configuration | Config 1 |  | DLBWP.0.1 | DLBWP.0.1 | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1 |  | DLBWP.1.1 | DLBWP.1.1 | DLBWP.1.1 |
| UL dedicated BWP configuration | Config 1 |  | ULBWP.1.1 | ULBWP.1.1 | ULBWP.1.1 |
| SRS configuration | Config 1 |  | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 |
| PDSCH Reference measurement channel | Config 1 |  | SR.2.1 TDD | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET parameters | Config 1 |  | CR.2.1 TDD | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET parameters | Config 1 |  | CCR.2.1 TDD | CCR.2.1 TDD | CCR.2.1 TDD |
| OCNG Patterns |  |  | OP.1 | OP.1 | OP.1 |
| SMTC Configuration |  |  | SMTC.1 | SMTC.1 | SMTC.1 |
| SSB Configuration | Config 1 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 2x2 Low | 2x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| NocNote 2 |  | dBm/15 kHz | -104 | -104 | -104 |
| SS-RSRP Note 3 |  | dBm/SCS | -84 | -87 | -87 |
| Ês/Iot |  | dB | 17 | 14 | 14 |
| Ês/Noc |  | dB | 17 | 14 | 14 |
| NocNote 2 | Config 1 | dBm/SCS | -104 | -101 | -101 |
| IoNote3 | Config 1 | dBm/9.36 MHz | -58.96 | - | - |
|  |  | dBm/ 38.16MHz | - | -55.79 | -55.79 |
| Time offset to Cell1 Note 5 |  | s | - | 0 | 0 |
| Propagation Condition |  |  | AWGN | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |  |

##### A.6.5.7B.2.2	Test Requirements

The UE behaviour follows the requirements defined in clause 8.2.2.2.10B.

UE shall send L1-RSRP report while meeting the accuracy requirements defined in clause 10.1.19.2.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.6.5.7C	DL interruptions at switching between two uplink bands with two transmit antenna connectors

#### A.6.5.7C.1	DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA

##### A.6.5.7C.1.1	Test Purpose and Environment

The purpose of this test is to verify DL interruption requirements during UE dynamic switching between two uplink bands with two transmit antenna connectors defined in clause 8.2.2.2.10C. The test case is applicable for an uplink band pair of an inter-band FDD-TDD CA configuration when the capability [uplinkTxSwitchingPeriod2T2T] is present, where NR UL carrier 1 in band A is capable of two transmit antenna connector, NR UL carrier 2 and carrier 3 in band B are capable of two transmit antenna connectors. NR UL carrier 2 and carrier 3 are two contiguous aggregated carriers, and band A and band B are different bands with different carrier frequencies.

There are three cells: FR1 FDD PCell (Cell 1), FR1 TDD SCell (Cell 2) and FR1 TDD SCell (Cell 3) where cell 1 in band A is with 2Tx, cell2 and cell 3 in band B with 2Tx, cell2 and cell3 are two contiguous aggregated carriers. The test parameters for the three cells are given in Table A.6.5.7C.1.1-1, Table A.6.5.7C.1.1-2 and Table A.6.5.7C.1.1-3 below.

For NR FDD carrier (Cell 1), aperiodic CSI-RS for L1-RSRP reporting is triggered with power boosting [6dB] on the following symbol in the slot overlapping with the special slot of the NR TDD carrier (Cell 2) and NR TDD carrier (Cell 3):

-	symbol#12 if UE does not report uplinkTxSwitching-DL-Interruption;

-	otherwise,

-	symbol #8 if UE capability [uplinkTxSwitchingPeriod2T2T]  is 210us or

-	symbol #9 if UE capability [uplinkTxSwitchingPeriod2T2T] is 140us or

-	symbol #10 if UE capability [uplinkTxSwitchingPeriod2T2T] is 35us.

For NR TDD carrier (Cell 2) and NR TDD carrier (Cell 3), aperiodic CSI-RS for L1-RSRP reporting is configured with power boosting [6dB] on the following symbol in the special slot:

-	symbol#10 if UE does not report [uplinkTxSwitching-DL-Interruption];

-	otherwise,

-	symbol #4 if UE capability [uplinkTxSwitchingPeriod2T2T] is 210us or

-	symbol #5 if UE capability [uplinkTxSwitchingPeriod2T2T] is 140us or

-	symbol #8 if UE capability [uplinkTxSwitchingPeriod2T2T] is 35us.

This test verifies that the UE correctly report the L1-RSRP reporting. The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, [uplinkTxSwitchingPeriod2T2T] is indicated to UE.

Table A.6.5.7C.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | NR Cell 1: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode NR Cell 2: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR Cell 3: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.6.5.7C.1.1-2: General test parameters for DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | Config 1 | 1, 2, 3 | Three radio channels are used for this test. |
| Active cell |  | Config 1 | Cell 1: FR1 PCell Cell 2: FR1 SCell Cell 3: FR1 SCell | FR1 PCell on RF channel number 1 FR1 SCell on RF channel number 2 FR1 SCell on RF channel number 3 |
| CP length |  | Config 1 | Normal |  |
| DRX |  | Config 1 | OFF |  |
| Measurement gap pattern Id |  | Config 1 | OFF |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| CSI-RS configuration for L1-RSRP reporting |  | Config 1 | Cell 1: CSI-RS.1.5 FDD Cell 2: CSI-RS.2.5 TDD Cell 3: CSI-RS.2.5 TDD |  |
| T1 | s | Config 1 | 5 |  |

Table A.6.5.7C.1.1-3: Cell specific test parameters for DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA

| Parameter |  | Unit | Cell1 | Cell2 | Cell3 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR1 | FR1 | FR1 |
| Duplex mode | Config 1 |  | FDD | TDD | TDD |
| TDD configuration | Config 1 |  | N/A | TDDConf.2.1 except that: S=’11DL: 1GP:2UL’; nrofDownlinkSymbols: 11 nrofUplinkSymbols: 2 | TDDConf.2.1 except that: S=’11DL: 1GP:2UL’; nrofDownlinkSymbols: 11 nrofUplinkSymbols: 2 |
| BWchannel | Config 1 |  | 10 MHz: NRB,c = 52 | 40 MHz: NRB,c = 106 | 40 MHz: NRB,c = 106 |
| Initial BWP Configuration | Config 1 |  | DLBWP.0.1 | DLBWP.0.1 | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1 |  | DLBWP.1.1 | DLBWP.1.1 | DLBWP.1.1 |
| UL dedicated BWP configuration | Config 1 |  | ULBWP.1.1 | ULBWP.1.1 | ULBWP.1.1 |
| SRS configuration | Config 1 |  | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0resourceMappingnrofSymbols: n2 | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD | CCR.2.1 TDD | CCR.2.1 TDD |
| OCNG Patterns |  |  | OP.1 | OP.1 | OP.1 |
| SMTC Configuration |  |  | SMTC.1 | SMTC.1 | SMTC.1 |
| SSB Configuration | Config 1 |  | SSB.1 FR1 | SSB.2 FR1 | SSB.2 FR1 |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low | 2x2 Low | 2x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| NocNote 2 |  | dBm/15 kHz | -104 | -104 | -104 |
| SS-RSRP Note 3 |  | dBm/SCS | -87 | -87 | -87 |
| Ês/Iot |  | dB | 17 | 14 | 14 |
| Ês/Noc |  | dB | 17 | 14 | 14 |
| NocNote 2 | Config 1 | dBm/SCS | -104 | -101 | -101 |
| IoNote3 | Config 1 | dBm/9.36 MHz | -58.96 | - | - |
|  |  | dBm/ 38.16MHz | - | -55.79 | -55.79 |
| Time offset to Cell1 Note 5 |  | s | - | 0 | 0 |
| Propagation Condition |  |  | AWGN | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |  |

##### A.6.5.7C.1.2	Test Requirements

The UE behaviour follows the requirements defined in clause 8.2.2.2.10C.

UE shall send L1-RSRP report while meeting the accuracy requirements defined in clause 10.1.19.2.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.7C.2	DL interruptions at switching between two uplink bands with two transmit antenna connectors in TDD-TDD CA

##### A.6.5.7C.2.1	Test Purpose and Environment

The purpose of this test is to verify DL interruption requirements during UE dynamic switching between two uplink carriers defined in clause 8.2.2.2.10C. The test case is applicable for an uplink band pair of an inter-band TDD-TDD CA configuration when the capability [uplinkTxSwitchingPeriod2T2T] is present, , where NR UL carrier 1 in band A is capable of two transmit antenna connector, NR UL carrier 2 and carrier 3 in band B are capable of two transmit antenna connectors. NR UL carrier 2 and carrier 3 are two contiguous aggregated carriers, and band A and band B are different bands with different carrier frequencies.

There are three cells: FR1 TDD PCell (Cell 1), FR1 TDD SCell (Cell 2) and FR1 TDD SCell (Cell 3) where cell 1 in band A is with 2Tx, cell2 and cell 3 in band B with 2Tx, cell2 and cell3 are two contiguous aggregated carriers. The test parameters for the three cells are given in Table A.6.5.7C.2.1-1, Table A.6.5.7C.2.1-2 and Table A.6.5.7C.2.1-3 below.

For NR TDD PCell (Cell 1), aperiodic CSI-RS for L1-RSRP reporting is triggered with power boosting [6dB] on the following symbol in the special slot:

-	symbol#10 if UE does not report uplinkTxSwitching-DL-Interruption;

-	otherwise,

-	symbol #4 if UE capability [uplinkTxSwitchingPeriod2T2T] is 210us or

-	symbol #5 if UE capability [uplinkTxSwitchingPeriod2T2T] is 140us or

-	symbol #8 if UE capability [uplinkTxSwitchingPeriod2T2T] is 35us.

For NR TDD SCell (Cell 2) and NR TDD SCell (Cell 3), aperiodic CSI-RS for L1-RSRP reporting is configured with power boosting [6dB] on the following symbol on the 2nd special slot of every 8 slots:

-	symbol#10 if UE does not report [uplinkTxSwitching-DL-Interruption];

-	otherwise,

-	symbol #4 if UE capability [uplinkTxSwitchingPeriod2T2T] is 210us or

-	symbol #5 if UE capability [uplinkTxSwitchingPeriod2T2T] is 140us or

-	symbol #8 if UE capability [uplinkTxSwitchingPeriod2T2T] is 35us.

This test verifies that the UE correctly report the L1-RSRP reporting. The test case is only applicable to UE which supports simultaneousRxTxInterBandCA.

The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, [uplinkTxSwitchingPeriod2T2T] is indicated to UE.

Table A.6.5.7C.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | NR Cell 1: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR Cell 2: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR Cell 3: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.6.5.7C.2.1-2: General test parameters for DL interruptions at switching between two uplink bands with two transmit antenna connectors in TDD-TDD CA

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | Config 1 | 1, 2, 3 | Three radio channels are used for this test. |
| Active cell |  | Config 1 | Cell 1: FR1 PCell Cell 2: FR1 SCell Cell 3: FR1 SCell | FR1 PCell on RF channel number 1 FR1 SCell on RF channel number 2 FR1 SCell on RF channel number 3 |
| CP length |  | Config 1 | Normal |  |
| DRX |  | Config 1 | OFF |  |
| Measurement gap pattern Id |  | Config 1 | OFF |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| CSI-RS configuration for L1-RSRP reporting |  | Config 1 | Cell 1: CSI-RS.2.5 TDD Cell 2: CSI-RS.2.5 TDD Cell 3: CSI-RS.2.5 TDD |  |
| T1 | s | Config 1 | 5 |  |

Table A.6.5.7C.2.1-3: Cell specific test parameters for DL interruptions at switching between two uplink bands with two transmit antenna connectors in TDD-TDD CA

| Parameter |  | Unit | Cell1 | Cell2 | Cell3 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR1 | FR1 | FR1 |
| Duplex mode | Config 1 |  | TDD | TDD | TDD |
| TDD configuration | Config 1 |  | TDDConf.2.1 except that S=’1 1DL: :2UL’; nrofDownlinkSymbols: 11 nrofUplinkSymbols: 2 | TDDConf.2.2 | TDDConf.2.2 |
| BWchannel | Config 1 |  | 40 MHz: NRB,c = 106 | 40 MHz: NRB,c = 106 | 40 MHz: NRB,c = 106 |
| Initial BWP Configuration | Config 1 |  | DLBWP.0.1 | DLBWP.0.1 | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1 |  | DLBWP.1.1 | DLBWP.1.1 | DLBWP.1.1 |
| UL dedicated BWP configuration | Config 1 |  | ULBWP.1.1 | ULBWP.1.1 | ULBWP.1.1 |
| SRS configuration | Config 1 |  | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 |
| PDSCH Reference measurement channel | Config 1 |  | SR.2.1 TDD | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET parameters | Config 1 |  | CR.2.1 TDD | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET parameters | Config 1 |  | CCR.2.1 TDD | CCR.2.1 TDD | CCR.2.1 TDD |
| OCNG Patterns |  |  | OP.1 | OP.1 | OP.1 |
| SMTC Configuration |  |  | SMTC.1 | SMTC.1 | SMTC.1 |
| SSB Configuration | Config 1 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low | 2x2 Low | 2x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| NocNote 2 |  | dBm/15 kHz | -104 | -104 | -104 |
| SS-RSRP Note 3 |  | dBm/SCS | -84 | -87 | -87 |
| Ês/Iot |  | dB | 17 | 14 | 14 |
| Ês/Noc |  | dB | 17 | 14 | 14 |
| NocNote 2 | Config 1 | dBm/SCS | -104 | -101 | -101 |
| IoNote3 | Config 1 | dBm/9.36 MHz | -58.96 | - | - |
|  |  | dBm/ 38.16MHz | - | -55.79 | -55.79 |
| Time offset to Cell1 Note 5 |  | s | - | 0 | 0 |
| Propagation Condition |  |  | AWGN | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |  |

##### A.6.5.7C.2.2	Test Requirements

The UE behaviour follows the requirements defined in clause 8.2.2.2.10C.

UE shall send L1-RSRP report while meeting the accuracy requirements defined in clause 10.1.19.2.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.6.5.8	UE specific CBW change

#### A.6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX

##### A.6.5.8.1.1	Test Purpose and Environment

The purpose of this test is to verify the UE specific CBW change delay requirement defined in clause 8.13.

The supported test configurations are shown in Table A.6.5.8.1.1-1. The test scenario comprises of one Cell (Cell 1), which is PCell as given in Table A.6.5.8.1.1-2. Cell-specific parameters are specified in Table A.6.5.8.1.1-3.

PDCCHs indicating new transmissions shall be sent continuously on Cell 1 to ensure that the UE sends ACK/NACK during the test.

Before the test starts:

- UE is connected to Cell 1 (PCell) on radio channel 1.

- UE has bandwidth part BWP-1 in its RRC-configuration for Cell 1 (PCell).

- UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 of initial condition in PCell.

- UE has been configured with UE specific CBW (CBW-1).

- UE is indicated in SCS-SpecificCarrier [2] that the UE specific CBW is CBW-1 as the initial condition in Cell 1 (PCell).

Cell1 (PCell) has constant signal levels throughout the test.

The test consists of 1 time period, with duration of T1.

During T1,

Time period T1 starts when a RRCReconfiguration containing SCS-SpecificCarrier with updated UE specific CBW, sent from the test equipment to the UE, is completely received at the UE side in PCell’s slot # denoted i. The UE shall reconfigure its UE specific CBW with the updated CBW-2 for the final condition.

The UE shall be able to receive PDSCH on PCell from the first DL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length as defined in clause 8.13 and starts to report valid ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length+k1 on the PCell’s BWP-1 on CBW-2 for the final condition. The UE shall be continuously scheduled on the PCell’s BWP-1 on CBW-2  for the final condition starting from the first DL slot right after slot i+TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length.

TRRCprocessingDelay and TCBWchangeDelayRRC are defined in clause 8.13.

The test equipment verifies the UE specific CBW switching delay in PCell by estimating the time from the moment the RRC Reconfiguration message including updated UE specific CBW configuration is sent until the moment a vaild ACK/NACK is received.

Table A.6.5.8.1.1-1: Supported test configurations for UE specific CBW change in SA scenario

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.5.8.1.1-2: General test parameters for UE specific CBW change in SA scenario

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active Cell |  | Cell 1 | Cell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| T1 | s | 0.2 |  |

Table A.6.5.8.1.1-3: NR Cell specific test parameters for UE specific CBW change in SA scenario

| Parameter |  |  |  | Unit | Cell 1 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  |  |  | FR1 |
| Duplex mode |  | Config 1 |  |  | FDD |
|  |  | Config 2,3 |  |  | TDD |
| TDD configuration |  | Config 1 |  |  | Not Applicable |
|  |  | Config 2 |  |  | TDDConf.1.1 |
|  |  | Config 3 |  |  | TDDConf.2.1 |
| BWchannel |  | Config 1 |  |  | 10 MHz: NRB,c = 52 |
|  |  | Config 2 |  |  | 10 MHz: NRB,c = 52 |
|  |  | Config 3 |  |  | 40 MHz: NRB,c = 106 |
| Active DL BWP ID |  | Config 1,2, 3 |  |  | 1 |
| Initial DL BWP Configuration (BWP-1) |  | Config 1,2, 3 |  |  | DLBWP.0.2 |
| Initial UL BWP Configuration |  | Config 1,2, 3 |  |  | ULBWP.0.2 |
| Initial Condition | Active DLCBW-1 Configureation | Config 1, 2, 3 |  |  | DLCBW.1.1 |
|  | Active UL CBW-1 Configuration | Config 1, 2, 3 |  |  | ULCBW.1.1 |
| Final Condition | Active DLCBW-1 Configureation | Config 1, 2, 3 |  |  | DLCBW.1.2 |
|  | Active UL CBW-1 Configuration | Config 1, 2, 3 |  |  | ULCBW.1.2 |
| PDSCH Reference measurement channel |  | Config 1 |  |  | SR.1.1 FDD |
|  |  | Config 2 |  |  | SR.1.1 TDD |
|  |  | Config 3 |  |  | SR2.1 TDD |
| RMSI CORESET parameters |  | Config 1 |  |  | CR.1.1 FDD |
|  |  | Config 2 |  |  | CR.1.1 TDD |
|  |  | Config 3 |  |  | CR2.1 TDD |
| Dedicated CORESET parameters |  | Config 1 |  |  | CCR.1.1 FDD |
|  |  | Config 2 |  |  | CCR.1.1 TDD |
|  |  | Config 3 |  |  | CCR.2.1 TDD |
| OCNG Patterns |  |  |  |  | OP.1 |
| SSB Configuration |  | Config 1,2 |  |  | SSB.1 FR1 |
|  |  | Config 3 |  |  | SSB.2 FR1 |
| SMTC Configuration |  |  |  |  | SMTC.1 |
| TRS Configuration |  |  | Config 1 |  | TRS.1.1 FDD |
|  |  |  | Config 2 |  | TRS.1.1 TDD |
|  |  |  | Config 3 |  | TRS.1.2 TDD |
| Antenna Configuration |  |  |  |  | 1x2 Low |
| Propagation Condition |  |  |  |  | AWGN |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS(Note 1) |  |  |  |  |  |
| NocNote 2 |  | Config 1,2 |  | dBm/SCS | -104 |
|  |  | Config 3 |  |  | -101 |
| SS-RSRP Note 3 |  | Config 1,2 |  | dBm/SCS | -87 |
|  |  | Config 3 |  |  | -84 |
| Ês/Iot |  |  |  | dB | 17 |
| Ês/Noc |  |  |  | dB | 17 |
| IoNote3 |  | Config 1,2 |  | dBm/ 9.36MHz | -58.96 |
|  |  | Config 3 |  | dBm/ 38.16MHz | -52.86 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.1 is linked with ULBWP.0.1; DLBWP.1.1 is linked with ULBWP.1.1; as defined in clause 12 of TS 38.213 [3]. |  |  |  |  |  |

##### A.6.5.8.1.2	Test Requirements

During T1, the UE shall be ready for the reception of uplink grant for the PCell from the first DL slot that occurs right after the begining of slot i+TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length and starts to report valid ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length+k1.

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [7].

All of the above test requirements shall be fulfilled in order for the observed UE specific CBW change delay on the PCell to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.6.5.9	Pathloss reference signal switching delay

#### A.6.5.9.1	MAC-CE based pathloss reference signal switch delay

##### A.6.5.9.1.1	Test Purpose and Environment

The purpose of this test is to verify the MAC-CE based pathloss reference signal switch delay requirement defined in clause 8.14.

The supported test configurations are shown in Table A.6.5.9.1.1-1. The test scenario comprises of one cell (Cell 1) as given in Table A.6.5.9.1.1-2. Cell-specific parameters of the cell are specified in Table A.6.5.9.1.1-3 below.

The test consists of 3 successive time periods, with duration of T1, T2 and T3, respectively.

Prior to the start of the time duration T1,

-	UE is connected to Cell 1 on radio channel 1.

-	UE shall be fully synchronized to SSB #0.

During T1,

-	The UE shall track SSB #1 so that SSB #1 as a pathloss reference signal is known to the UE.

Time period T2 starts when the UE is configured of the power headroom reporting functionality by upper layers by the test equipment and the UE shall transmit a PHR during T2.

During T2,

-	UE is configured with a phr-ProhibitTimer timer value for Cell 1.

-	UE is configured with a phr-Tx-PowerFactorChange value for Cell 1.

During T3,

Time period T3 starts when a PDSCH carrying MAC-CE activation for pathloss reference signal switch, sent from the test equipment to the UE to swicth the pathloss reference signal from SSB 0 to SSB 1, is received at the UE side in Cell1’s slot # denoted i. The UE shall switch its pathloss reference signal to the target one and send PHR.

The UE shall be able to apply the target pathloss reference signal of the serving cell on which pathloss reference signal switch occurs no later than the slot i + THARQ+ 3ms + 5*Ttarget_PL-RS + 2msNR slot length as defined in clause 8.14.  The UE shall be able to apply old pathloss reference signals until the slot i + THARQ+ 3Nslotsubframe,µ as defined in clause 8.14.

The test equipment verifies the pathloss RS switch time by counting the slots from the time when the pathloss RS switch command is transmitted till a PHR is received during T3.

Table A.6.5.9.1.1-1: MAC-CE based pathloss reference signal switch supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.5.9.1.1-2: General test parameters for MAC-CE based pathloss reference signal switch in SA

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active PCell |  |  | Cell 1 |  |
| RF Channel Number |  |  | 1 |  |
| Duplex mode | Config 1 |  | FDD |  |
|  | Config 2, 3 |  | TDD |  |
| DL initial BWP configuration | Config 1, 2, 3 |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration | Config 1, 2, 3 |  | DLBWP.1.1 |  |
| UL initial BWP configuration | Config 1, 2, 3 |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration | Config 1, 2, 3 |  | ULBWP.1.1 |  |
| TDD Configuration | Config 1 |  | Not Applicable |  |
|  | Config 2 |  | TDDConf.1.1 |  |
|  | Config 3 |  | TDDConf.2.1 |  |
| CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| SSB Configuration | Config 1 |  | SSB.1 FR1 |  |
|  | Config 2 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB.2 FR1 |  |
| SMTC Configuration | Config 1, 2 |  | SMTC.1 |  |
|  | Config 3 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1, 2 |  | 15 kHz |  |
|  | Config 3 |  | 30 kHz |  |
| SSB index assigned as pathloss RS |  |  | 0 in T1, 0 in T2, 1 in T3 |  |
| OCNG parameters |  |  | OP.1 |  |
| CP length |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |  |
| DRX |  |  | OFF |  |
| Gap pattern ID |  |  | gp0 |  |
| phr-ProhibitTimer |  | sub frame | 0 |  |
| phr-Tx-PowerFactorChange |  | dB | 5 |  |
| phr-PeriodicTimer |  | sub frame | infinity |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| T1 |  | s | [2] |  |
| T2 |  | s | [2] |  |
| T3 |  | s | 0.2 |  |

Table A.6.5.9.1.1-3: NR Cell specific test parameters for MAC-CE based pathloss reference signal switch in SA

| Parameter |  |  |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 | T3 |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  | dB | 0 |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  | dB |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  | dB |  |  |  |
| SSB with index 0 |  |  |  | dB | [7] |  |  |
|  |  | Config 1, 2, 3 |  | dBm/15kHz | [-101] |  |  |
|  |  |  |  | dB | [7] |  |  |
|  | SS-RSRP Note 4 |  | Config 1, 2 | dBm/ SCS | [-94] |  |  |
|  |  |  | Config 3 |  | [-91] |  |  |
| SSB with index 1 |  |  |  | dB | [-3] |  |  |
|  |  | Config 1, 2, 3 |  | dBm/15kHz | [-101] |  |  |
|  |  |  |  | dB | [-3] |  |  |
|  | SS-RSRP Note 4 |  | Config 1, 2 | dBm/ SCS | [-104] |  |  |
|  |  |  | Config 3 |  | [-101] |  |  |
| Io Note 5 | Config 1, 2 |  |  | dBm | -65.3/9.36MHz |  |  |
|  | Config 3 |  |  |  | -59.2/38.16MHz |  |  |
| Propagation condition |  |  |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. Note 5: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. |  |  |  |  |  |  |  |

##### A.6.5.9.1.2	Test Requirements

During T3, the UE shall start to send the PHR for PCell no later than the slot i + THARQ+ 3ms + 5*Ttarget_PL-RS + 2msNR slot length.

During T3, the UE shall start to send the PHR for PCell no earlier than the slot i + THARQ+ 3Nslotsubframe,µ.

Where, THARQ is the timing between pathloss reference MAC-CE activation command and acknowledgement as specified in [7], Ttarget_PL-RS is the periodicity of the target pathloss reference signal which is SSB in this test.

During T3, UE shall send L1-RSRP report with measurement results for both SSB0 and SSB1.

All of the above test requirements shall be fulfilled in order for the observed pathloss RS switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

Note: The UE shall be given proper uplink transmission grant during T2 and T3.

## A.6.6	Measurement procedure

### A.6.6.1	Intra-frequency Measurements

#### A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX

##### A.6.6.1.1.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2.5.1 and 9.2.5.2.

##### A.6.6.1.1.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell and neighbour cell are given in Table A.6.6.1.1.1-1 and A.6.6.1.1.1-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

Table A.6.6.1.1.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.1.1.2-2: General test parameters for SA intra-frequency event triggered reporting without gap for FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |

Table A.6.6.1.1.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for FR1

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | TN/A |  | TN/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  | N/A |  |
|  |  | 3 | CR.2.1 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  | N/A |  |
|  |  | 3 | CCR.2.1 TDD |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | --58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.1.1.3	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.1.2	SA event triggered reporting tests without gap under DRX

##### A.6.6.1.2.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2.5.1 and 9.2.5.2.

##### A.6.6.1.2.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell are given in Table A.6.6.1.2.2-1, A.6.6.1.2.2-2 and A.6.6.1.2.2-3 below. In the measurement controlinformation, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.6.6.1.2.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.1.2.2-2: General test parameters for SA intra-frequency event triggered reporting without gap for PCell in FR1 with DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1, 2, 3 | Cell 1 |  |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 |  | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |  |
|  |  | 2 | SSB.1 FR1 |  |  |
|  |  | 3 | SSB.2 FR1 |  |  |
| SMTC configuration |  | 1 | SMTC.2 |  |  |
|  |  | 2 | SMTC.1 |  |  |
|  |  | 3 | SMTC.1 |  |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |  |
| CP length |  | 1, 2, 3 | Normal |  |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |  |
| Filter coefficient |  | 1, 2, 3 | 0 |  | L3 filtering is not used |
| DRX |  | 1, 2, 3 | DRX.1 | DRX. 7 |  |
| Time offset between serving and neighbour cells |  | 1 | 3 ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s |  | Synchronous cells |
|  |  | 3 | 3 s |  | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |  |
| T2 | s | 1, 2, 3 | 5 | 10 |  |

Table A.6.6.1.2.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for PCell in FR1 with DRX

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | TN/A |  | TN/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  | N/A |  |
|  |  | 3 | CR.2.1 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  | N/A |  |
|  |  | 3 | CCR.2.1 TDD |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | --58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.1.2.3	Test Requirements

In test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

In test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 6400 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX

##### A.6.6.1.3.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clause 9.2.6.2 and 9.2.6.3.

##### A.6.6.1.3.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell are given in Table A.6.6.1.3.1-1 and A.6.6.1.3.1-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

Table A.6.6.1.3.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.1.3.2-2: General test parameters for SA intra-frequency event triggered reporting with per-UE gaps for PCell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| Measurement gap type |  | 1, 2, 3 | Per-UE gaps |  |
| Measurement gap repitition periodicity | ms | 1, 2, 3 | 40 |  |
| Measurement gap length | ms | 1, 2, 3 | 6 |  |
| Measurement gap offset | ms | 1, 2, 3 | 39 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CSI-RS parameters |  | 1 | CSI-RS.1.2 FDD resource #0 |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX | ms | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |

Table A.6.6.1.3.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting with per-UE gaps for PCell in FR1

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | TN/A |  | TN/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  | N/A |  |
|  |  | 3 | CR.2.1 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.2 FDD |  | N/A |  |
|  |  | 2 | CCR.1.2 TDD |  | N/A |  |
|  |  | 3 | CCR.2.1 TDD |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | CSI-RS |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | --58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.1.3.3	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX

##### A.6.6.1.4.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clause 9.2.6.2 and 9.2.6.3.

##### A.6.6.1.4.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell are given in Table A.6.6.1.4.2-1, A.6.6.1.4.2-2 and A.6.6.1.4.2-3 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.6.6.1.4.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.1.4.2-2: General test parameters for SA intra-frequency event triggered reporting with per-UE gaps for PCell in FR1 with DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1, 2, 3 | Cell 1 |  |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 |  | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |  |
| Measurement gap type |  | 1, 2, 3 | Per-UE gaps |  |  |
| Measurement gap repitition periodicity | ms | 1, 2, 3 | 40 |  |  |
| Measurement gap length | ms | 1, 2, 3 | 6 |  |  |
| Measurement gap offset | ms | 1, 2, 3 | 39 |  |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |  |
|  |  | 2 | SSB.1 FR1 |  |  |
|  |  | 3 | SSB.2 FR1 |  |  |
| SMTC configuration |  | 1 | SMTC.2 |  |  |
|  |  | 2 | SMTC.1 |  |  |
|  |  | 3 | SMTC.1 |  |  |
| CSI-RS parameters |  | 1 | CSI-RS.1.2 FDD resource #0 |  |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |  |
| CP length |  | 1, 2, 3 | Normal |  |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |  |
| Filter coefficient |  | 1, 2, 3 | 0 |  | L3 filtering is not used |
| DRX |  | 1, 2, 3 | DRX.1 | DRX. 7 |  |
| Time offset between serving and neighbour cells |  | 1 | 3 ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s |  | Synchronous cells |
|  |  | 3 | 3 s |  | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |  |
| T2 | s | 1, 2, 3 | 5 | 10 |  |

Table A.6.6.1.4.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting with per-UE gaps for PCell in FR1 with DRX

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | TN/A |  | TN/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  | N/A |  |
|  |  | 3 | CR.2.1 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.2 FDD |  | N/A |  |
|  |  | 2 | CCR.1.2 TDD |  | N/A |  |
|  |  | 3 | CCR.2.1 TDD |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | CSI-RS |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | --58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: Table A.6.6.1.4.2-1The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Table A.6.6.1.4.2-1Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Table A.6.6.1.4.2-1SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.1.4.2-4: Void

Table A.6.6.1.4.2-5: Void

##### A.6.6.1.4.3	Test Requirements

In test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

In test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 6400 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading

##### A.6.6.1.5.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the FDD intra-frequency cell search requirements in clause 9.2.5.1 and 9.2.5.2.

##### A.6.6.1.5.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for FDD PCell and neighbour cell are given in Table A.6.6.1.5.2-1 and A.6.6.1.5.2-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

Table A.6.6.1.5.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |

Table A.6.6.1.5.2-2: General test parameters for SA intra-frequency event triggered reporting without gap for FDD PCell in FR1 with SSB index reading

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1 | Cell 1 |  |
| Neighbour cell |  | 1 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
| A3-Offset | dB | 1 | -4.5 |  |
| CP length |  | 1 | Normal |  |
| Hysteresis | dB | 1 | 0 |  |
| Time To Trigger | s | 1 | 0 |  |
| Filter coefficient |  | 1 | 0 | L3 filtering is not used |
| DRX | ms | 1 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
| T1 | s | 1 | 5 |  |
| T2 | s | 1 | 5 |  |

Table A.6.6.1.5.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for FDD PCell in FR1 with SSB index reading

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
| OCNG Patterns |  | 1 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
| IInitial BWP configuration |  | 1 | DLBWP.0,1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | --64.60 | -62.25 |
| Propagation Condition |  | 1 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.1.5.3	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE is required to read the neighbour cell SSB index and report the acquired SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading

##### A.6.6.1.6.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the FDD intra-frequency cell search requirements in clause 9.2.6.2 and 9.2.6.3.

##### A.6.6.1.6.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for FDD PCell and neighbour cell are given in Table A.6.6.1.6.2-1 and A.6.6.1.6.2-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

Table A.6.6.1.6.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |

Table A.6.6.1.6.2-2: General test parameters for SA intra-frequency event triggered reporting with gap for FDD PCell in FR1 with SSB index reading

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1 | Cell 1 |  |
| Neighbour cell |  | 1 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1 | 1: Cell 1 and Cell 2 |  |
| Measurement gap type |  | 1 | Per-UE gaps |  |
| Measurement gap repitition periodicity | ms | 1 | 40 |  |
| Measurement gap length | ms | 1 | 6 |  |
| Measurement gap offset | ms | 1 | 39 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
| CSI-RS parameters |  | 1 | CSI-RS.1.2 FDD resource #0 |  |
| A3-Offset | dB | 1 | -4.5 |  |
| CP length |  | 1 | Normal |  |
| Hysteresis | dB | 1 | 0 |  |
| Time To Trigger | s | 1 | 0 |  |
| Filter coefficient |  | 1 | 0 | L3 filtering is not used |
| DRX | ms | 1 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
| T1 | s | 1 | 5 |  |
| T2 | s | 1 | 5 |  |

Table A.6.6.1.6.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting with gap for FDD PCell in FR1 with SSB index reading

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.2 FDD |  | N/A |  |
| OCNG Patterns |  | 1 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
| IInitial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1 | CSI-RS |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | --64.60 | -62.25 |
| Propagation Condition |  | 1 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.1.6.3	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE is required to read the neighbour cell SSB index and report the acquired SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.1.7	SA event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16

##### A.6.6.1.7.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event for UE configured with highSpeedMeasFlag-r16. This test will partly verify the intra-frequency cell search requirements in clauses 9.2.5.1 and 9.2.5.2.

##### A.6.6.1.7.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell are given in Table A.6.6.1.7.2-1, A.6.6.1.7.2-2 and A.6.6.1.7.2-3 below. In the measurement controlinformation, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.6.6.1.7.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.1.7.2-2: General test parameters for SA intra-frequency event triggered reporting without gap for PCell in FR1 with DRX for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| highSpeedMeasFlag-r16 |  | 1,2,3 | Present | To enable high speed measurement enhancements |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 | DRX.7 | 640ms DRX cycle |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 6 |  |

Table A.6.6.1.7.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for PCell in FR1 with DRX for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 |  | T2 |
| TDD configuration |  | 1 | TN/A |  | TN/A |  |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |  |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |  |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity |  | -1.46 |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity |  | 4 |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity |  | -94 |
|  |  | 2 | -94 | -94 | -Infinity |  | -94 |
|  |  | 3 | -91 | -91 | -Infinity |  | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | -64.60 |  | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | -64.60 |  | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | -58.50 |  | -56.16 |
| Propagation Condition |  | 1, 2 | AWGN |  |  | AWGN 1944Hz Note 4 |  |
|  |  | 3 | AWGN |  |  | AWGN 3334Hz Note 5 |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The AWGN 1944 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 1944Hz. Note 5: The AWGN 3334 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 3334Hz. |  |  |  |  |  |  |  |

##### A.6.6.1.7.3	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 5120 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.1.8	SA event triggered reporting tests without gap under DRX for UE configured with highSpeedMeasCA-Scell-r17

##### A.6.6.1.8.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event for UE configured with highSpeedMeasCA-Scell-r17. This test will partly verify the intra-frequency cell search requirements in clause 9.2.5.1 and 9.2.5.2.

##### A.6.6.1.8.2	Test parameters

Three cells are deployed in the test, which are FR1 PCell (Cell 1), a FR1 deactivated SCell (Cell 2) and a FR1 neighbour cell (Cell 3) on the same frequency as the SCell (Cell 2). The test parameters for PCell are given in Table A.6.6.1.8.21, A.6.6.1.8.2-2 and A.6.6.1.8.2-3 below. In the measurement control information, a measurement object is configured for the frequency of the SCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 3.

UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.6.6.1.8.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.1.8.2-2: General test parameters for SA intra-frequency event triggered reporting without gap for deactivated SCell in FR1 with DRX highSpeedMeasCA-Scell-r17

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| highSpeedMeasCA-Scell-r17 |  | 1,2,3 | Present | To enable high speed measurement enhancements |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Deactivated SCell |  | 1, 2, 3 | Cell 2 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 3 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 2: Cell 2 and 3 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 | DRX.6 |  |
| measCycleSCell |  | 1, 2, 3 | 640ms |  |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 3 is 3ms later than the timing of Cell 2. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 10 |  |

Table A.6.6.1.8.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for deactivated SCell in FR1 with DRX highSpeedMeasCA-Scell-r17

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TDD configuration |  | 1 | N/A |  | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  | N/A |  | N/A |  |
|  |  | 3 | CR.2.1 TDD |  | N/A |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  | N/A |  | N/A |  |
|  |  | 3 | CCR.2.1 TDD |  | N/A |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
|  | dB | 1 | 4 | 4 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | -58.50 | -56.16 | -58.50 | -56.16 |
| Propagation Condition |  | 1,2 | AWGN |  |  |  | AWGN 1944Hz Note 4 |  |
|  |  | 3 | AWGN |  |  |  | AWGN 3334Hz Note 5 |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The AWGN 1944 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 1944Hz. Note 5: The AWGN 3334 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 3334Hz. |  |  |  |  |  |  |  |  |

##### A.6.6.1.8.3	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 5760 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.6.6.2	Inter-frequency Measurements

#### A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used

##### A.6.6.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2.  The test parameters are given in Tables A.6.6.2.1.1-1, A.6.6.2.1.1-2 and A.6.6.2.1.1-3.

In test 1 measurement gap pattern configuration # 0 as defined in Table A.6.6.2.1.1-2 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table A.6.6.2.1.1-2 is provided for UE that supports per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 2. Otherwise it is only required to pass test 1.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Table A.6.6.2.1.1-1: SA event triggered reporting tests without SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.2.1.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 | 4 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 9 | 9 |  |
| SMTC-SSB parameters |  | Config 1 | SSB.1 FR1 |  | As specified in clause A.3.10.1 |
|  |  | Config 2 | SSB.1 FR1 |  | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.2 FR1 |  | As specified in clause A.3.10.1 |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 1 | 1 |  |

Table A.6.6.2.1.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  | -98 |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |  |
|  |  |  | Config 3 | -95 |  | -95 |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 |  | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 |  | -Infinity | -88 |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 |  | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 |  | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 |  | -70.05 | -62.26 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 |  | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |

##### A.6.6.2.1.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 760 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2 UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used

##### A.6.6.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2.  The test parameters are given in Tables A.6.6.2.2.1-1, A.6.6.2.2.1-2 and A.6.6.2.2.1-3.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table A.6.6.2.2.1-2 is provided for UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #4 as defined in Table A.6.6.2.2.1-2 is provided for UE that supports per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

Table A.6.6.2.2.1-1: SA event triggered reporting tests without SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.2.2.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  |  |  | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  |  |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 |  |  |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 |  | 4 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 9 |  | 9 |  |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.1 | DRX. 7 | DRX.1 | DRX. 7 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  |  |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |  |  |
| T2 | s | Config 1,2,3 | 1.1 | 11 | 1.1 | 11 |  |

Table A.6.6.2.2.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  |  | T1 | T2 |  |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  |  | 2 |  |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  |  | NA |  |  |
|  | Initial UL BWP |  | Config 1, 2, 3 | ULBWP.0.1 |  |  |  | NA |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |  | NA |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |  | NA |  |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  |  | NA |  |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  |  | NA |  |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  |  | NA |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  |  | OP.1 |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  | NA |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  | NA |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  | NA |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  | NA |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  | NA |  |  |
|  |  |  | Config 3 | CR.2.1 TDD |  |  |  | NA |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  |  |  | NA |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  | NA |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  | NA |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  |  | SSB.6 FR1 |  |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  |  |  | SMTC.5 |  |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  |  | SMTC.4 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2,3 | -98 |  | -98 |  |  |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |  |  |  |
|  |  |  | Config 3 | -95 |  | -95 |  |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 |  |  | -Infinity | -91 |  |
|  |  |  | Config 3 | -91 | -91 |  |  | -Infinity | -88 |  |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 |  |  | -Infinity | 7 |  |
|  |  | dB | Config 1,2,3 | 4 | 4 |  |  | -Infinity | 7 |  |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 |  |  | -70.05 | -62.2 |  |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 |  |  | -63.94 | -56.15 |  |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |

Table A.6.6.2.2.1-4: DRX-Configuration for SA inter-frequency event triggered reporting without SSB time index detection

| Field | Test1&3 | Test2&4 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| drx-onDurationTimer | ms1 | ms1 | As specified in clause 6.3.2 in TS 38.331 [2] |
| drx-InactivityTimer | ms1 | ms1 |  |
| drx-RetransmissionTimerDL | sl1 | sl1 |  |
| drx-RetransmissionTimerUL | sl1 | sl1 |  |
| drx-LongCycleStartOffset | ms40 | Ms640 |  |
| shortDRX | disable | disable |  |

Table A.6.6.2.2.1-5: TimeAlignmentTimer -Configuration SA inter-frequency event triggered reporting without SSB time index detection

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | ms500 | As specified in clause 6.3.2 in TS 38.331 [2] |

##### A.6.6.2.2.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1080 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 10240 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 3 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1080 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 4 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 10240 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1, 2, 3 and 4 UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.2.3	Void

#### A.6.6.2.4	Void

#### A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used

##### A.6.6.2.5.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2. The test parameters are given in Tables A.6.6.2.5.1-1, A.6.6.2.5.1-2 and A.6.6.2.5.1-3.

In test 1 measurement gap pattern configuration # 0 as defined in Table A.6.6.2.5.1-2 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table A.6.6.2.5.1-2 is provided for UE that supports per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 2. Otherwise it is only required to pass test 1.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Table A.6.6.2.5.1-1: SA event triggered reporting tests with SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.2.5.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 with SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 | 4 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 9 | 9 |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 1.1 | 1 |  |

Table A.6.6.2.5.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 with SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.2 |
|  |  | dBm/38.16MHz | Config 3 | -58.4 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

##### A.6.6.2.5.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1040 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 880 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2 UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used

##### A.6.6.2.6.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2. The test parameters are given in Tables A.6.6.2.6.1-1, A.6.6.2.6.1-2 and A.6.6.2.6.1-3.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table A.6.6.2.6.1-2 is provided for UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #4 as defined in Table A.6.6.2.6.1-2 is provided for UE that supports per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

Table A.6.6.2.6.1-1: SA event triggered reporting tests with SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.2.6.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 with SSB time index detection

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  |  |  | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  |  |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 |  |  |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 |  | 4 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 9 |  | 9 |  |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.1 | DRX. 7 | DRX.1 | DRX. 7 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1 | 3 ms |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3 s |  |  |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |  |  |
| T2 | s | Config 1,2,3 | 1.3 | 13.5 | 1.3 | 13.5 |  |

Table A.6.6.2.6.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 with SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

##### A.6.6.2.6.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1280 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 12160 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 3 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1280 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 4 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 12160 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1, 2, 3 and 4 UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.2.7	Void

#### A.6.6.2.8	Void

#### A.6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern

##### A.6.6.2.9.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event when mandatory gap pattern with 3ms MGL is configured.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2.  The test parameters are given in Tables A.6.6.2.9.1-1, A.6.6.2.9.1-2 and A.6.6.2.9.1-3.

In test 1 measurement gap pattern configuration # 3 as defined in Table A.6.6.2.9.1-2 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #2 as defined in Table A.6.6.2.9.1-2 is provided for UE that supports per-FR gap. If a UE supports per-FR gap and gap pattern configuration #2, it is only required to pass test 2. Otherwise it is only required to pass test 1.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Table A.6.6.2.9.1-1: SA event triggered reporting tests without SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.2.9.1-2: General test parameters for SA inter-frequency event triggered reporting with additional mandatory gap pattern

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 3 | 2 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 9 | 9 |  |
| SMTC-SSB parameters |  | Config 1 | SSB.1 FR1 |  | As specified in clause A.3.10.1 |
|  |  | Config 2 | SSB.1 FR1 |  | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.2 FR1 |  | As specified in clause A.3.10.1 |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 1 | 1 |  |

Table A.6.6.2.9.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting with additional mandatory gap pattern

| Parameter |  | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.1 |  |  | SMTC.4 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  | -98 |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |  |
|  |  |  | Config 3 | -95 |  | -95 |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 |  | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 |  | -Infinity | -88 |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 |  | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 |  | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 |  | -70.05 | -62.26 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 |  | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |

##### A.6.6.2.9.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1280 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2 UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used

##### A.6.6.2.10.1	Test Purpose and Environment

The purpose of this test is to verify that the UE which supports interFrequencyMeas-Nogap-r16 makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search without measurement gap requirements in clause 9.3.9.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on RF channel 2. The SSB of cell 2 is completely within UE’s active BWP BW. The RBs containing SSB from cell 1 and cell 2 should be different in frequency location within the cell bandwidth. The test parameters are given in Tables A.6.6.2.10.1-1, A.6.6.2.10.1-2 and A.6.6.2.10.1-3.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

Table A.6.6.2.10.1-1: SA event triggered reporting tests when DRX is used for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.2.10.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 when DRX is used

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 | NR cell 2 is on NR RF channel number 2. |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.1 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 1 |  |

Table A.6.6.2.10.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 when DRX is used

| Parameter |  | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  |  | T1 | T2 |  |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  |  | 2 |  |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  |  | NA |  |  |
|  | Initial UL BWP |  | Config 1, 2, 3 | ULBWP.0.1 |  |  |  | NA |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |  | NA |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |  | NA |  |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  |  | NA |  |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  |  | NA |  |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  |  | NA |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  |  | OP.1 |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  | NA |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  | NA |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  | NA |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  | NA |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  | NA |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  | NA |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  |  | SSB.6 FR1 |  |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  |  |  | SMTC.5 |  |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  |  | SMTC.4 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2,3 | -98 |  | -98 |  |  |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |  |  |  |
|  |  |  | Config 3 | -95 |  | -95 |  |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 |  |  | -Infinity | -91 |  |
|  |  |  | Config 3 | -91 | -91 |  |  | -Infinity | -88 |  |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 |  |  | -Infinity | 7 |  |
|  |  | dB | Config 1,2,3 | 4 | 4 |  |  | -Infinity | 7 |  |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 |  |  | -70.05 | -62.2 |  |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 |  |  | -63.94 | -56.15 |  |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |

Table A.6.6.2.10.1-4: TimeAlignmentTimer -Configuration SA inter-frequency event triggered reporting when DRX is used

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | ms500 | As specified in clause 6.3.2 in TS 38.331 [2] |

##### A.6.6.2.10.2	Test Requirements

In test config 1, UE is required to report SSB time index. UE is not required to report SSB time index. The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1080 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test config 2 and 3, UE is not required to report SSB time index. The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 900 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.2.11	SA event triggered reporting tests for FR1 without gap when DRX is not used

##### A.6.6.2.11.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.9.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2.  The SSB of Cell 2 is completely within UE’s active BWP BW. The RBs containing SSB from cell 1 and cell 2 should be different in frequency location within the cell bandwidth. The test parameters are given in Tables A.6.6.2.11.1-1, A.6.6.2.11.1-2 and A.6.6.2.11.1-3.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Table A.6.6.2.11.1-1: SA event triggered reporting tests without SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.2.11.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 without gap

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 | NR cell 2 is on NR RF channel number 2. |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 1 |  |

Table A.6.6.2.11.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without gap

| Parameter |  | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  | -98 |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |  |
|  |  |  | Config 3 | -95 |  | -95 |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 |  | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 |  | -Infinity | -88 |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 |  | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 |  | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 |  | -70.05 | -62.26 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 |  | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |

##### A.6.6.2.11.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

The UE is not required to read the neighbour cell SSB index in this test.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.2.12	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for UE configured with highSpeedMeasInterFreq-r17

##### A.6.6.2.12.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event when UE is configured with highSpeedMeasInterFreq-r17. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2.  The test parameters are given in Tables A.6.6.2.12.1-1, A.6.6.2.12.1-2 and A.6.6.2.12.1-3.

In test 1 measurement gap pattern configuration # 0 as defined in Table A.6.6.2.12.1-2 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table A.6.6.2.12.1-2 is provided for UE that supports per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 1. Otherwise it is only required to pass test 1.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

Table A.6.6.2.12.1-1: SA event triggered reporting tests without SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.2.12.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection for UE configured with highSpeedMeasInterFreq-r17

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 | 4 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 9 | 9 |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.4 |  | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 2.3 | 2.3 |  |

Table A.6.6.2.12.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection for UE configured with highSpeedMeasInterFreq-r17

| Parameter |  | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  |  | T1 | T2 |  |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  |  | 2 |  |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  |  | NA |  |  |
|  | Initial UL BWP |  | Config 1, 2, 3 | ULBWP.0.1 |  |  |  | NA |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |  | NA |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |  | NA |  |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  |  | NA |  |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  |  | NA |  |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  |  | NA |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  |  | OP.1 |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  | NA |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  | NA |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  | NA |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  | NA |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  | NA |  |  |
|  |  |  | Config 3 | CR.2.1 TDD |  |  |  | NA |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  |  |  | NA |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  | NA |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  | NA |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  |  | SSB.6 FR1 |  |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  |  |  | SMTC.5 |  |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  |  | SMTC.4 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2,3 | -98 |  | -98 |  |  |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |  |  |  |
|  |  |  | Config 3 | -95 |  | -95 |  |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 |  |  | -Infinity | -91 |  |
|  |  |  | Config 3 | -91 | -91 |  |  | -Infinity | -88 |  |
|  |  | dB | Config 1,2,3 | 4 | 4 |  |  | -Infinity | 7 |  |
|  |  | dB | Config 1,2,3 | 4 | 4 |  |  | -Infinity | 7 |  |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 |  |  | -70.05 | -62.2 |  |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 |  |  | -63.94 | -56.15 |  |
| Propagation Condition |  |  | Config 1,2 | AWGN |  |  | AWGN 1944Hz Note 5 |  |  |  |
|  |  |  | Config 3 | AWGN |  |  | AWGN 3334Hz Note 6 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The AWGN 1944 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 1944Hz. Note 6: The AWGN 3334 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 3334Hz. |  |  |  |  |  |  |  |  |  |  |

##### A.6.6.2.12.2	Test Requirements

In test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 2240 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 2240 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1&2, UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.6.6.3	Inter-RAT Measurements

#### A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1

##### A.6.6.3.1.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements when operating in standalone (SA) operation with PCell in FR1. This test shall partly verify the cell search and measurement requirements in Clauses 9.4.2 and 9.4.3.

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell. In the measurement control information from the PCell it is indictated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2.

Supported test configurations are shown in table A.6.6.3.1.1-1. General test parameters are provided in Table A.6.6.3.1.1-2 below. Test parameters for Cell 1 and Cell 2, valid for both time duration T1 and T2, are provided in Tables A.6.6.3.1.1-3 and A.6.6.3.1.1-4, respectively.

Table A.6.6.3.1.1-1: Supported test configurations in SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.6.3.1.1-2: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 1 | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in Tables A.6.6.3.1.1-2 and A.6.6.3.1.1-3. |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 | As specified in Clause Table 9.1.2-1. Per-UE gap pattern. |
| NR measurement quantity |  | SS-RSRP | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | Note 1 | SS-RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| b2-Threshold2EUTRA | dBm | -95 | E-UTRAN RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| Note 1: Values are defined in Table A.6.6.3.1.1-3 |  |  |  |

Table A.6.6.3.1.1-3: PCell specific test parameters for SA inter-RAT E-UTRA event triggered reporting in non-DRX with PCell in FR1

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 2, 3 | FDD |  |
|  |  |  |  | 4, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |
|  |  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORSET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORSET reference channel |  |  |  | 1, 4 | CCR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CCR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  |  | 3, 6 | TRS.1.2 TDD |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | --96 |  |
|  |  |  |  | 3, 6 | --93 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -104 |  |
|  |  |  |  | 3, 6 | -101 |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 16 | 0 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 16 | 0 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -104 |
|  |  |  |  | 3, 6 | -85 | -101 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -104 |
|  |  |  |  | 3, 6 | -85 | -101 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -59.94 | -73.04 |
|  |  |  | dBm/38.16 MHz | 3, 6 | -53.84 | -66.93 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.3.1.1-4: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -73.21+10log (NRB,c /50) | -56.12+10log (NRB,c /50) |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.6.6.3.1.2	Test Requirements

The UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 3.84s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1

##### A.6.6.3.2.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements when operating in standalone (SA) operation with PCell in FR1 when DRX is used. This test shall partly verify the cell search and measurement requirements in Clauses 9.4.2 and 9.4.3. There are two test cases. In test 1 the UE shall be configured with DRX cycle of 40 ms. In test 2 the UE shall be configured with DRX cycle of 640 ms.

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell. In the measurement control information from the PCell it is indctated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2.

In each test the UE shall be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furthermore the UE shall be allocated with PUSCH resource at every DRX cycle.

Supported test configurations are shown in table A.6.6.3.2.1-1. General test parameters are provided in Table A.6.6.3.2.1-2 below. Test parameters for Cell 1 and Cell 2, valid for both time duration T1 and T2, are provided in Tables A.6.6.3.2.1-3 and A.6.6.3.2.1-4, respectively.

Table A.6.6.3.2.1-1: Supported test configurations in SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.6.3.2.1-2: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1

| Parameter | Unit | Test 1 | Test 2 | Comment |
| --- | --- | --- | --- | --- |
|  |  | Value |  |  |
| NR RF Channel Number |  | 1 |  | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 |  | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in Tables A.6.6.3.2.1-2 and A.6.6.3.2.1-3. |  |  |
| Active cell |  | Cell 1 |  | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 |  | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 |  | As specified in Clause Table 9.1.2-1. Per-UE gap pattern. |
| NR measurement quantity |  | SS-RSRP |  | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP |  | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | Note 1 |  | SS-RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| b2-Threshold2EUTRA | dBm | -95 |  | E-UTRAN RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| Hysteresis | dB | 0 |  |  |
| TimeToTrigger | s | 0 |  |  |
| Filter coefficient |  | 0 |  | L3 filtering is not used |
| DRX |  | DRX.1 | DRX. 7 | DRX cycle configurations DRX.1 and DRX. 7 are defined in Table A.3.3.1-1 and Table A.3.3. 7-1 respectively. |
| T1 | s | 5 |  |  |
| T2 | s | 5 | 15 |  |
| Note 1: Values are defined in Table A.6.6.3.2.1-3 |  |  |  |  |

Table A.6.6.3.2.1-3: PCell specific test parameters for SA inter-RAT E-UTRA event triggered reporting in DRX with PCell in FR1

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 2, 3 | FDD |  |
|  |  |  |  | 4, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |
|  |  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORSET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORSET reference channel |  |  |  | 1, 4 | CCR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CCR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  |  | 3, 6 | TRS.1.2 TDD |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | -96 |  |
|  |  |  |  | 3, 6 | -93 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -104 |  |
|  |  |  |  | 3, 6 | -101 |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 16 | 0 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 16 | 0 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -104 |
|  |  |  |  | 3, 6 | -85 | -101 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -104 |
|  |  |  |  | 3, 6 | -85 | -101 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -59.94 | -73.04 |
|  |  |  | dBm/38.16 MHz | 3, 6 | -53.84 | -66.93 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.3.2.1-4: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -73.21+10log (NRB,c /50) | -56.12+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

##### A.6.6.3.2.2	Test Requirements

In test 1, the UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 3.84s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

In test 2, the UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 12.8s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for UE configured with highSpeedMeasFlag-r16

##### A.6.6.3.3.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements for UE configured with highSpeedMeasFlag-r16 in standalone (SA) operation with PCell in FR1 when DRX is used. This test shall partly verify the cell search and measurement requirements in Clauses 9.4.2 and 9.4.3.

In the test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell. In the measurement control information from the PCell it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2.

The UE shall be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore the UE shall be allocated with PUSCH resource at every DRX cycle.

Supported test configurations are shown in table A.6.6.3.3.1-1. General test parameters are provided in Table A.6.6.3.3.1-2 below. Test parameters for Cell 1 and Cell 2, valid for both time duration T1 and T2, are provided in Tables A.6.6.3.3.1-3 and A.6.6.3.3.1-4, respectively.

Table A.6.6.3.3.1-1: Supported test configurations in SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1 for UE configured with highSpeedMeasFlag-r16

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.6.3.3.1-2: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1 for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in Tables A.6.6.3.3.1-2 and A.6.6.3.3.1-3. |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 | As specified in Clause Table 9.1.2-1. Per-UE gap pattern. |
| NR measurement quantity |  | SS-RSRP | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | Note 1 | SS-RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| b2-Threshold2EUTRA | dBm | -97 | E-UTRAN RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | DRX.6 | DRX cycle configurations DRX.6 is defined in Table A.3.3.1-6. |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| Note 1: Values are defined in Table A.6.6.3.3.1-3 |  |  |  |

Table A.6.6.3.3.1-3: PCell specific test parameters for SA inter-RAT E-UTRA event triggered reporting in DRX with PCell in FR1 for UE configured with highSpeedMeasFlag-r16

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 2, 3 | FDD |  |
|  |  |  |  | 4, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |
|  |  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORESET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORESET reference channel |  |  |  | 1, 4 | CCR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CCR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  |  | 3, 6 | TRS.1.2 TDD |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | -98 |  |
|  |  |  |  | 3, 6 | -95 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -106 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -106 |  |
|  |  |  |  | 3, 6 | -103 |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 18 | -2 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 18 | -2 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -108 |
|  |  |  |  | 3, 6 | -85 | -105 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -108 |
|  |  |  |  | 3, 6 | -85 | -105 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -59.98 | -75.92 |
|  |  |  | dBm/38.16 MHz | 3, 6 | -53.88 | -69.82 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.3.3.1-4: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1 for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -106 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 19 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 19 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -78.22+10log (NRB,c /50) | -59.16+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2, 3, 4, 5, 6 | AWGN1944 |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

##### A.6.6.3.3.2	Test Requirements

In the test, the UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 4.8s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.6.6.4	L1-RSRP measurement for beam reporting

#### A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used

##### A.6.6.4.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.5.4.1, with the testing configurations for NR cells in Table A.6.6.4.1.1-1.

Table A.6.6.4.1.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.4.1.2	Test parameters

There is one cells in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.6.6.4.1.2-1 and Table A.6.6.4.1.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the SSBs and report periodically. The test consists of two successive time periods, with time duration of T1 and T2 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

Table A.6.6.4.1.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| OCNG Patterns | 1~3 |  | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | Off |
| reportConfigType | 1~3 |  | periodic |
| reportQuantity | 1~3 |  | ssb-Index-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| L1-RSRP reporting period | 1~3 | slot | 80 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 1 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.6.6.4.1.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -91.65 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -88.65 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.84 |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.4.1.3	Test Requirements

The UE shall send L1-RSRP report every 80 slots. No later than 640ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report including results of both SSB0 and SSB1 while meeting the absolute accuracy requirement in clause 10.1.19.1.1 and relative accuracy requirement in clause 10.1.19.1.2. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used

##### A.6.6.4.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.5.4.1, with the testing configurations for NR cells in Table A.6.6.4.2.1-1.

Table A.6.6.4.2.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.4.2.2	Test parameters

There is one cells in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.6.6.4.2.2-1 and Table A.6.6.4.2.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the SSBs and report periodically. The test consists of two successive time periods, with time duration of T1 and T2 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

Table A.6.6.4.2.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| OCNG Patterns | 1~3 |  | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | DRX.3 |
| reportConfigType | 1~3 |  | periodic |
| reportQuantity | 1~3 |  | ssb-Index-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| L1-RSRP reporting period | 1~3 | slot | 80 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 1 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.6.6.4.2.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -91.65 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -88.65 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.84 |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.4.2.3	Test Requirements

The UE shall send L1-RSRP report every 80 slots. No later than 640ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report including results of both SSB0 and SSB1 while meeting the absolute accuracy requirement in clause 10.1.19.1.1 and relative accuracy requirement in clause 10.1.19.1.2. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used

##### A.6.6.4.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.5.4.2, with the testing configurations for NR cells in Table A.6.6.4.3.1-1.

Table A.6.6.4.3.1-1: Applicable NR configurations for FR1 CSI-RS based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.4.3.2	Test parameters

There is one cells in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.6.6.4.3.2-1 and Table A.6.6.4.3.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the CSI-RS and report aperiodically. The test consists of a single time period T1, during which the UE is triggered via DCI to report L1-RSRP on aperiodic CSI-RS resources. UE is also configured to measure L1-RSRP based on SSB. After 80ms from the beginning of the test, the DCI trigger comes in slot n (0 for Config 1,2 and 8 for Config 3) of a frame and UE provides the report back based on the reporting configuration as defined in Table A.6.6.4.3.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM and BFD based on the SSBs.

Table A.6.6.4.3.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| CSI-RS configuration | 1 |  | CSI-RS 1.3 FDD |
|  | 2 |  | CSI-RS 1.3 TDD |
|  | 3 |  | CSI-RS 2.3 TDD |
| OCNG Patterns | 1~3 |  | OP.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| DRX configuration | 1~3 |  | Off |
| reportConfigType | 1~3 |  | aperiodic |
| reportQuantity | 1~3 |  | cri-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| qcl-Info | 1~3 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1~3 | slots | 8 |
| T1 | 1~3 | s | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.6.6.4.3.2-2: CSI-RS specific test parameters

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Note1 | 1~3 | dBm/15kHz | -94.65 |  |
| Note1 | 1,2 | dBm/SSB SCS | -94.65 |  |
|  | 3 |  | -91.65 |  |
|  | 1~3 | dB | 0 | 3 |
| CSI-RS RSRP Note2 | 1,2 | dBm/SSB SCS | -94.65 | -91.65 |
|  | 3 |  | -91.65 | -88.65 |
| Io Note2 | 1,2 | dBm/9.36 MHz | -63.69 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -55.84 |
|  | 1~3 | dB | 0 | 3 |
| Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.6.6.4.3.3	Test Requirements

After 80ms from the beginning of the test, the UE shall send L1-RSRP report at slot 8  from the reception of DCI triggering the L1-RSRP measurement. The L1-RSRP report shall include the results for both CSI-RS#0 and CSI-RS#1 while meeting the absolute accuracy requirement in clause 10.1.20.1.1 and relative accuracy requirement in clause 10.1.20.1.2.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used

##### A.6.6.4.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.5.4.2, with the testing configurations for NR cells in Table A.6.6.4.4.1-1.

Table A.6.6.4.4.1-1: Applicable NR configurations for FR1 CSI-RS based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.4.4.2	Test parameters

There is one cells in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.6.6.4.4.2-1 and Table A.6.6.4.4.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the CSI-RS and report aperiodically. The test consists of a single time period T1, during which the UE is triggered via DCI to report L1-RSRP on aperiodic CSI-RS resources. UE is also configured to measure L1-RSRP based on SSB. After 80ms from the beginning of the test, the DCI trigger comes in slot n (0 for Config 1,2 and 8 for Config 3) of a frame and UE provides the report back based on the reporting configuration as defined in Table A.6.6.4.4.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM and BFD based on the SSBs.

Table A.6.6.4.4.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| CSI-RS configuration | 1 |  | CSI-RS 1.3 FDD |
|  | 2 |  | CSI-RS 1.3 TDD |
|  | 3 |  | CSI-RS 2.3 TDD |
| OCNG Patterns | 1~3 |  | OP.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| DRX configuration | 1~3 |  | DRX.3 |
| reportConfigType | 1~3 |  | aperiodic |
| reportQuantity | 1~3 |  | cri-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| qcl-Info | 1~3 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1~3 | slots | 8 |
| T1 | 1~3 | s | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.6.6.4.4.2-2: CSI-RS specific test parameters

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Note1 | 1~3 | dBm/15kHz | -94.65 |  |
| Note1 | 1,2 | dBm/SSB SCS | -94.65 |  |
|  | 3 |  | -91.65 |  |
|  | 1~3 | dB | 0 | 3 |
| CSI-RS RSRP Note2 | 1,2 | dBm/SSB SCS | -94.65 | -91.65 |
|  | 3 |  | -91.65 | -88.65 |
| Io Note2 | 1,2 | dBm/9.36 MHz | -63.69 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -55.84 |
|  | 1~3 | dB | 0 | 3 |
| Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.6.6.4.4.3	Test Requirements

After 80ms from the beginning of the test, the UE shall send L1-RSRP report at slot 8  from the reception of DCI triggering the L1-RSRP measurement. The L1-RSRP report shall include the results for both CSI-RS#0 and CSI-RS#1 while meeting the absolute accuracy requirement in clause 10.1.20.1.1 and relative accuracy requirement in clause 10.1.20.1.2.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16

##### A.6.6.4.5.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement when UE is configured with highSpeedMeasFlag-r16. This test will partly verify the L1-RSRP measurement requirements for UE configured with highSpeedMeasFlag-r16 in clause 9.5.4.1, with the testing configurations for NR cells in Table A.6.6.4.5.1-1.

Table A.6.6.4.5.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.4.5.2	Test parameters

There is one cells in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.6.6.4.5.2-1 and Table A.6.6.4.5.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the SSBs and report periodically. The test consists of two successive time periods, with time duration of T1 and T2 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

Table A.6.6.4.5.2-1: General test parameters for UE configured with highSpeedMeasFlag-r16

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| OCNG Patterns | 1~3 |  | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | DRX.3 |
| reportConfigType | 1~3 |  | periodic |
| reportQuantity | 1~3 |  | ssb-Index-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| L1-RSRP reporting period | 1~3 | slot | 80 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 2 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~2 |  | AWGN 1944 Hz |
|  | 3 |  | AWGN 3334 Hz |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.6.6.4.5.2-2: SSB specific test parameters for UE configured with highSpeedMeasFlag-r16

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -91.65 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -88.65 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.84 |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.4.5.3	Test Requirements

The UE shall send L1-RSRP report every 80 slots. No later than 640ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report including results of both SSB0 and SSB1 while meeting the absolute accuracy requirement in clause 10.1.19.1.1 and relative accuracy requirement in clause 10.1.19.1.2. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.4.6	Inter-cell SSB based L1-RSRP measurements on FR1 PCell when DRX is used

##### A.6.6.4.6.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement for SSB of a cell with additional PCI. This test will partly verify the L1-RSRP measurement requirements in clause 9.13.4.1, with the testing configurations for NR serving cells in Table A.6.6.4.6.1-1.

Table A.6.6.4.6.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.4.6.2	Test parameters

There are two cells in the test, the FR1 PCell (Cell 1) and a cell with additional PCI (Cell 2). The test parameters for the Cell 1 are given in Table A.6.6.4.6.2-1. The test parameters for Cell 2 are given in Table A.6.6.4.6.2-2 below.

SSB#0 and SSB#1 are transmitted on Cell 1 and Cell 2. In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the SSB#0 and report measurement results periodically. The test consists of two successive time periods, with time duration of T1 and T2 respectively. At the beginning of T2, SSB#1 starts transmission and the UE is configured for L1-RSRP measurement on SSB#1. The test has higher layer parameter timeRestrictionForChannelMeasurements configured in CSI-ReportConfig and additionalPCIList configured in CSI-SSB-ResourceSet.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the the SSB#0 for Cell 1.

Table A.6.6.4.6.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| Active cell | 1~3 |  | Cell 1 |
| A cell with additional PCI | 1~3 |  | Cell 2 |
| RF Channel Number | 1~3 |  | 1: Cell 1 and Cell 2 |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 for Cell1 and Cell2 |
|  | 2 |  | SSB.3 FR1 for Cell1 and Cell2 |
|  | 3 |  | SSB.4 FR1 for Cell1 and Cell2 |
| OCNG Patterns | 1~3 |  | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | DRX.3 |
| reportConfigType | 1~3 |  | periodic |
| reportQuantity | 1~3 |  | ssb-Index-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| L1-RSRP reporting period | 1~3 | slot | 80 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 1 |
| CP length | 1~3 |  | Normal |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Time offset between serving and a cell with additional PCI | 1 |  | < CP |
|  | 2 |  | < CP |
|  | 3 |  | < CP |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.6.6.4.6.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 of Cell 1 |  | SSB#1 of Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -91.65 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -88.65 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.84 |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.4.6.3	Test Requirements

The UE shall send L1-RSRP report every 80 slots. No later than X ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report including results of SSB#1 while meeting the absolute accuracy requirement in clause 10.1.19.1.1 and relative accuracy requirement in clause 10.1.19.1.2, where X is

-	680 for Config 1&2

-	660 for Config 3

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.6.6.5	Inter-RAT UTRAN FDD measurements

#### A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1

##### A.6.6.5.1.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE makes correct event-triggered reporting of inter-RAT UTRAN FDD measurements when operating in standalone (SA) operation with PCell in FR1. This test shall partly verify the cell search and measurement requirements in Clause 9.4.6.

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT UTRAN FDD neighbour cell. In the measurement control information from the PCell it is indictated to the UE that event-triggered reporting with Event B1 (Inter RAT neighbour becomes better than threshold) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2.

Supported test configurations are shown in table A.6.6.5.1.1-1. General test parameters are provided in Table A.6.6.5.1.1-2 below. Test parameters for Cell 1 and Cell 2, valid for both time duration T1 and T2, are provided in Tables A.6.6.5.1.1-3 and A.6.6.5.1.1-4, respectively.

Table A.6.6.5.1.1-1: Supported test configurations in SA inter-RAT UTRAN FDD event triggered reporting in non-DRX with PCell in FR1

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, UTRA FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, UTRA FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, UTRA FDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.6.5.1.1-2: General test parameters for SA inter-RAT UTRAN FDD event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| UTRA RF Channel Number |  | 2 | 1 UTRA carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in Tables A.6.6.5.1.1-3 and A.6.6.5.1.1-4. |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 | As specified in Clause Table 9.1.2-1. Per-UE gap pattern. |
| Inter-RAT UTRA measurement quantity |  | CPICH Ec/Io | Measurement quantity for Cell 2 |
| b1-ThresholdUTRA-FDD | dB | -16.5 | CPICH Ec/Io threshold for SS-RSRP measurement on cell1 for event B1 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| Note 1: Values are defined in Table A.6.6.5.1.1-3 |  |  |  |

Table A.6.6.5.1.1-3: PCell specific test parameters for SA inter-RAT UTRAN FDD event triggered reporting in non-DRX with PCell in FR1

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3 | 1 |  |
| Duplex mode |  |  |  | 1, 2, 3 | FDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1 | 10: NRB,c = 52 (FDD) |  |
|  |  |  |  | 2 | 10: NRB,c = 52 (TDD) |  |
|  |  |  |  | 3 | 40: NRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1 | SR.1.1 FDD |  |
|  |  |  |  | 2 | SR.1.1 TDD |  |
|  |  |  |  | 3 | SR.2.1 TDD |  |
| CORESET reference channel |  |  |  | 1 | CR.1.1 FDD |  |
|  |  |  |  | 2 | CR.1.1 TDD |  |
|  |  |  |  | 3 | CR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2 | SSB.1 FR1 |  |
|  |  |  |  | 3 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1 | TRS.1.1 FDD |  |
|  |  |  |  | 2 | TRS.1.1 TDD |  |
|  |  |  |  | 3 | TRS.1.2 TDD |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3 | -106 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2 | -106 |  |
|  |  |  |  | 3 | -103 |  |
| Ês/Noc |  |  | dB | 1, 2, 3 | 18 | -2 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3 | 18 | -2 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2 | -88 | -108 |
|  |  |  |  | 3 | -85 | -105 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2 | -88 | -108 |
|  |  |  |  | 3 | -85 | -105 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2 | -59.98 | -75.92 |
|  |  |  | dBm/38.16 MHz | 3 | -53.88 | -69.82 |
| Propagation condition |  |  |  | 1, 2, 3 | ETDLA30 |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3 | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.5.1.1-4: UTRAN neighbour cell specific test parameters for SA inter-RAT UTRAN FDD event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| UTRA RF Channel Number |  | 2 |  |
| CPICH_Ec/Ior | dB | -10 |  |
| PCCPCH_Ec/Ior | dB | -12 |  |
| SCH_Ec/Ior | dB | -12 |  |
| PICH_Ec/Ior | dB | -15 |  |
| DPCH_Ec/Ior | dB | N/A |  |
| OCNS |  | -0.941 |  |
|  | dB | -Infinity | -1.8 |
|  | dBm/3.84 MHz | -70 |  |
| CPICH_Ec/Io | dB | -Infinity | -14 |
| Propagation Condition |  | AWGN |  |
| Note 1: The DPCH level is controlled by the power control loop. Note 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior. |  |  |  |

##### A.6.6.5.1.2	Test Requirements

The UE shall send one Event B1 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 2.4s from the start of period T2, i.e. when Cell 2 becomes detectable. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.6.6.6	CLI measurements

#### A.6.6.6.1	SRS-RSRP measurement with DRX

##### A.6.6.6.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of SRS-RSRP measurement. This test will verify the SRS-RSRP measurement requirements in clause 9.7.2.5 with the testing configurations for NR cells in Table A.6.6.6.1.1-1.

Table A.6.6.6.1.1-1: Applicable NR configurations for FR1 SRS-RSRP test

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SRS SCS, 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 30 kHz SRS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.6.1.2	Test Parameters

One cell is deployed in the test, which is FR1 PCell (Cell 1). The test parameters for PCell is given in Table A.6.6.6.1.2-1 and A.6.6.6.1.2-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event I1 is used. The test consists of two successive time periods, with time duration of T1 and T2, respectively.

During the test, the test system transmits SRS resource for measurement in the DL slot according to the SRS configuration in Table A.6.6.6.1.2-4 and the test parameters for the (virtual) neighbour cell UE in Table A. 6.6.6.1.2-3. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on SRS symbol to be transmitted and on 1 data symbol before SRS to be transmitted.

Table A.6.6.6.1.2-1: General test parameters for SRS-RSRP event triggered reporting for PCell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2 | Cell 1 |  |
| RF Channel Number |  | 1, 2 | 1: Cell 1 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.1 |  |
|  |  | 2 | SMTC.1 |  |
| SRS configuration |  | 1 | SRSConf.1 | Table A.6.6.6.1.2-3 |
|  |  | 2 | SRSConf.2 |  |
| CP length |  | 1, 2 | Normal |  |
| i1-Threshold | dBm | 1 | -97 |  |
|  |  | 2 | -95 |  |
| Hysteresis | dB | 1, 2 | 0 |  |
| Time To Trigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX |  | 1, 2 | DRX.7 |  |
| Time offset between DL from serving cell and SRS from test system | s | 1,2 | 17.67 |  |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | 5 |  |

Table A.6.6.6.1.2-2: NR Cell specific test parameters for SRS-RSRP event triggered reporting for PCell in FR1

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1 | TDDConf.1.1 |  |
|  |  | 2 | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 TDD |  |
|  |  | 2 | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 TDD |  |
|  |  | 2 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 TDD |  |
|  |  | 2 | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 TDD |  |
|  |  | 2 | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.1 |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |
|  |  | 2 |  |  |
| Note 2 | dBm/SCS | 1 | -98 |  |
|  |  | 2 | -95 |  |
| Propagation Condition |  | 1, 2 | AWGN |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |

Table A.6.6.6.1.2-3: NR Cell specific test parameters for SRS-RSRP event triggered reporting for neighbour cell UE

| Parameter | Unit | Test configuration | Neighbour cell UE |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| Note 2 | dBm/15 kHz | 1 | -98 |  |
|  |  | 2 |  |  |
| Note 2 | dBm/SCS | 1 | -98 |  |
|  |  | 2 | -95 |  |
|  | dB | 1 | -infinity | 4 |
|  |  | 2 |  |  |
|  | dB | 1 | -infinity | 4 |
|  |  | 2 |  |  |
| SRS-RSRP Note 3 | dBm/SCS kHz | 1 | -infinity | -94 |
|  |  | 2 | -infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -70.05 | -64.59 |
|  | dBm/38.16 MHz | 2 | -63.96 | -58.50 |
| Propagation Condition |  | 1, 2 | AWGN |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

Table A.6.6.6.1.2-4: SRS configuration for measurement reporting

|  | Field | SRSConf.1 | SRSConf.2 | Comments |
| --- | --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 0 |  |
|  | srs-ResourceIdList | 0 | 0 |  |
|  | resourceType | Periodic | Periodic |  |
|  | Usage | Codebook | Codebook |  |
| SRS-Resource | SRS-ResourceId | 0 | 0 |  |
|  | nrofSRS-Ports | Port1 | Port1 |  |
|  | transmissionComb | n2 | n2 |  |
|  | combOffset-n2 | 0 | 0 |  |
|  | cyclicShift-n2 | 0 | 0 |  |
|  | resourceMapping startPosition | 0 | 0 |  |
|  | resourceMapping nrofSymbols | n1 | n1 |  |
|  | resourceMapping repetitionFactor | n1 | n1 |  |
|  | freqDomainPosition | 0 | 0 |  |
|  | freqDomainShift | 0 | 0 |  |
|  | freqHopping c-SRS | 12 | 12 |  |
|  | freqHopping b-SRS | 0 | 0 |  |
|  | freqHopping b-hop | 0 | 0 |  |
|  | groupOrSequenceHopping | Neither | Neither |  |
|  | resourceType | Periodic | Periodic |  |
|  | periodicityAndOffset | sl640, 4 | sl640, 9 |  |
|  | sequenceId | 0 | 0 | Any 10 bit number |

##### A.6.6.6.1.3	Test Requirements

The UE shall send one Event I1 triggered measurement report, with a measurement reporting delay less than 1920 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.6.2	CLI-RSSI measurement with DRX

##### A.6.6.6.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of CLI-RSSI measurement. This test will verify the CLI-RSSI measurement requirements in clause 9.7.3.5 with the testing configurations for NR cells in Table A.6.6.6.2.1-1.

Table A.6.6.6.2.1-1: Applicable NR configurations for FR1 CLI-RSSI test

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SCS, 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 30 kHz SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.6.2.2	Test Parameters

One cell is deployed in the test, which are FR1 PCell (Cell 1). The test parameters for PCell is given in Table A.6.6.6.2.2-1 and A.6.6.6.2.2-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event I1 is used. The test consists of two successive time periods, with time duration of T1 and T2, respectively.

During the test, the test system does not transmit PDCCH/PDSCH/OCNG on symbols for CLI-RSSI measurement resource and on 1 data symbol before. The CLI-RSSI measurement resource configuration is in Table A.6.6.6.2.2-3.

Table A.6.6.6.2.2-1: General test parameters for CLI-RSSI event triggered reporting for PCell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2 | NR Cell 1 |  |
| RF Channel Number |  | 1, 2 | 1: Cell 1 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.1 |  |
|  |  | 2 | SMTC.1 |  |
| CLI-RSSI configuration |  | 1 | CLI-RSSIConf.1 | Table A.6.6.6.2.2-3 |
|  |  | 2 | CLI-RSSIConf.2 |  |
| CP length |  | 1, 2 | Normal |  |
| i1-Threshold | dBm | 1 | -93 |  |
|  |  | 2 | -93 |  |
| Hysteresis | dB | 1, 2 | 0 |  |
| Time To Trigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX |  | 1, 2 | DRX.7 |  |
| Time offset between DL from serving cell and OCNG from test system | s | 1,2 | 17.67 |  |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | 2 |  |

Table A.6.6.6.2.2-2: NR Cell specific test parameters for CLI-RSSI event triggered reporting for PCell in FR1

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1 | TDDConf.1.1 |  |
|  |  | 2 | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 TDD |  |
|  |  | 2 | SR.2.1 TDD |  |
| PUSCH parameters |  | 1 | N/A |  |
|  |  | 2 |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 TDD |  |
|  |  | 2 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 TDD |  |
|  |  | 2 | CCR.2.1 TDD |  |
| OCNG Patterns Note 3 |  | 1, 2 | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 TDD |  |
|  |  | 2 | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.1 |  |
| on CLI-RSSI measurement resource Note 2 | dBm/15 kHz | 1 | -116 | -108 |
|  |  | 2 |  |  |
| on CLI-RSSI measurement resource Note 2 | dBm/SCS | 1 | -116 | -108 |
|  |  | 2 | -113 | -105 |
| Io on CLI-RSSI measurement resource | dBm/9.36 MHz | 1 | -88.05 | -80.05 |
|  | dBm/38.16 MHz | 2 | -81.96 | -74.00 |
| Io on CLI-RSSI measurement resource | dBm/1.08 MHz | 1 | -97.43 | -89.43 |
|  | dBm/1.08 MHz | 2 | -97.44 | -89.44 |
| Propagation Condition |  | 1, 2 | AWGN |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: OCNG is not transmitted in the CLI-RSSI measurement resources. |  |  |  |  |

Table A.6.6.6.2.2-3: CLI-RSSI measurement resource configuration for measurement reporting

|  | Field | CLI-RSSIConf.1 | CLI-RSSIConf.2 |
| --- | --- | --- | --- |
| RSSI-Resource | rssi-ResourceId | 0 | 0 |
|  | rssi-SCS | 15 | 30 |
|  | startPRB | 0 | 0 |
|  | nrofPRBs | 52 | 106 |
|  | startPosition | 3 | 3 |
|  | nrofSymbols | 11 | 11 |
|  | rssi-PeriodicityAndOffset | sl640, 4 | sl640, 9 |

##### A.6.6.6.2.3	Test Requirements

The UE shall send one Event I1 triggered measurement report, with a measurement reporting delay less than 640 ms from the beginning of time period T2. The nominal RSSI used to evaluate the requirement shall be based on Io.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.6.6.7	NR measurements with autonomous gaps

#### A.6.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1

##### A.6.6.7.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of intra-frequency CGI identification of an NR neighbour cell in FR1 with autonomous gaps. This test shall partly verify the measurement requirements in Clause 9.11.

##### A.6.6.7.1.2	Test Parameters

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the FR1 PCell and Cell 2 is an FR1 neighbour cell on the same frequency as the PCell. The test parameters for PCell and neighbour cell are given in Table A.6.6.7.1.1-2 and A.6.6.7.1.3-2 below. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable.  A measurement object is configured for the frequency of the PCell and it is indicated to the UE that event-triggered reporting with Event A3 is used. The UE is expected to detect and send a measurement report with Event A3.

A new RRC message triggering CGI identification shall be sent to the UE during period T2, after the UE has reported Event A3. The RRC message shall create a measurement report configuration with purpose reportCGI and useAutonomousGaps set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying CGI identification is sent to the UE.

The test equipment verifies that potential interruption is carried out correctly by monitoring ACK/NACK sent in PCell during T3 untill a measurement report with CGI is sent.

Table A.6.6.71.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.7.1.2-2: General test parameters for SA intra-frequency CGI identification of NR neighbor cell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |

Table A.6.6.7.1.2-3: NR Cell specific test parameters for SA intra-frequency CGI identification of NR neighbor cell in FR1

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | TN/A |  | TN/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | --58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.7.1.3	Test Requirements

The UE shall send a measurement report containing the CGI of cell 2 within 252 ms from the start of time period T3.

Test requirement 	= RRC Procedure delay + Tidentify_CGI + reporting delay

= 10 + 240 + 2ms from the start of T3

= 252 ms

The UE shall be scheduled continuously throughout the test. From the start of T3 until 252 ms, the interruption on PCell shall not be more than the values specified for SA in clause 8.2.2.2.14.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA

##### A.6.6.7.2.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in NR SA in clause 9.4.7.

The test scenario comprises of one NR carrier and an E-UTRA carrier and two cells as given in tables A.6.6.7.2.1-1, A.6.6.7.2.1-2, A.6.6.7.2.1-3 and A.6.6.7.2.1-4. PDCCHs indicating new transmissions shall be sent continuously to ensure that the UE would have ACK/NACK sending during identifying a new CGI of E-UTRAN cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event B2. The RRC message shall create a measurement report configuration with purpose reportCGI and useAutonomousGaps set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE.

Table A.6.6.7.2.1-1: Supported test configurations of inter-RAT E-UTRAN cell using autonomous gap in SA

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.6.7.2.1-2: General test parameters for identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 | 1 LTE carrier frequency is used in the test |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| LTE Channel Bandwidth | MHz | 10 |  |
| LTE PDSCH/PCFICH/PDCCH/PHICH parameters |  |  | As specified in clause A.3.7.2.1 |
| CP length |  | Normal |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| useAutonomousGaps |  | TRUE | As specified in clause 5.5.3.1 in TS 38.331. |
| Time offset between cells | ms | 3 | Asynchronous cells |
| T1 | s | 5 |  |
| T2 | s | 10 |  |
| T3 | s | 5 |  |

Table A.6.6.7.2.1-3: PCell specific test parameters for identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR

| Parameter |  |  | Unit | Configuration | Cell 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |  | T3 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |  |  |
| Duplex mode |  |  |  | 1, 2, 3 | FDD |  |  |  |
|  |  |  |  | 4, 5, 6 | TDD |  |  |  |
| TDD Configuration |  | SCS=15 KHz |  | 2, 5 | TDDConf.1.1 |  |  |  |
|  |  | SCS=30 KHz |  | 3, 6 | TDDConf.2.1 |  |  |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |  |  |
|  |  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |  |  |
|  |  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |  |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |  |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |  |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |  |  |
| CORSET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |  |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |  |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |  |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |  |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |  |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |  |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |  |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |  |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |  |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |  |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |  |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | -98 |  |  |  |
|  |  |  |  | 3, 6 | -95 |  |  |  |
| b2-Threshold2EUTRA |  |  | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -109 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -106 |  |  |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -106 |  |  |  |
|  |  |  |  | 3, 6 | -103 |  |  |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 18 |  | -2 |  |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 18 |  | -2 |  |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 |  | -108 |  |
|  |  |  |  | 3, 6 | -85 |  | -105 |  |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 |  | -108 |  |
|  |  |  |  | 3, 6 | -85 |  | -105 |  |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -59.98 |  | -75.92 |  |
|  |  |  | dBm/38.16 MHz | 3, 6 | -53.88 |  | -69.82 |  |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |  |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

Table A.6.6.7.2.1-4: Cell specific test parameters for inter-RAT E-UTRAN cell for identification of a new CGI of E-UTRA cell using autonomous gaps

| Parameter | Unit | Configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |  |
|  |  | 4, 5, 6 | TDD |  |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 10 MHz: NRB,c = 50 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 10 MHz: R.3 FDD |  |  |
|  |  | 4, 5, 6 | 10 MHz: R.0 TDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 10 MHz: R.6 FDD |  |  |
|  |  | 4, 5, 6 | 10 MHz: R.6 TDD |  |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 10 MHz: OP.10 FDD |  |  |
|  |  | 4, 5, 6 | 10 MHz: OP.1 TDD |  |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -106 |  |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 | 7 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 | 7 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -99 | -99 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -99 | -99 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

##### A.6.6.7.2.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 2 within 200 milliseconds from the start of T3.

Test requirement = RRC Procedure delay with additional margin + Tidentify_CGI,E-UTRAN + reporting delay

= 15 + 30 + 150 + 2ms from the start of T3

= 197 ms, allow 200 ms.

-	The UE shall be scheduled continuously throughout the test, and from the start of T3 until 200 ms at least the number of ACK/NACK specified in NOTE 2 shall be detected as being transmitted by the UE.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

NOTE 2:	The overall ACK/NACK number is caused by two parts. Firstly, at least X ACK/NACK shall be sent during identifying the cell global identifier of cell 2, where X is defined in Table 8.2.2.2.15-1. Secondly, given that continuous DL data allocation, additional 43, 14 and 34 ACK/NACK shall be sent for FDD 15 kHz, TDD 15 kHz and TDD 30 kHz, respectively, from the start of T3 until 200 ms excludes 150 ms for identifying the cell global identifier of cell 2.

### A.6.6.8	L1-SINR measurement for beam reporting

#### A.6.6.8.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is used

##### A.6.6.8.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-SINR measurement. This test will partly verify the L1-SINR measurement requirements in clause 9.8.4.1, with the testing configurations for NR cells in Table A.6.6.8.1.1-1.

Table A.6.6.8.1.1-1: Applicable NR configurations for FR1 CSI-RS based L1-SINR test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.8.2	Test parameters

There is one cells in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.6.6.8.2-1 and Table A.6.6.8.2-2 below.

In the CSI-RS measurement configuration, UE is indicated to perform L1-SINR measurement on the CSI-RS and report aperiodically. The test consists of a single time period T1, during which the UE is triggered via DCI to report L1-SINR on aperiodic CSI-RS resources. After 80ms from the beginning of the test, the DCI trigger comes in slot n (1 Config 1,2 and 8 for Config 3) of a frame and UE provides the report back based on the reporting configuration as defined in Table A.6.6.8.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM and BFD based on the SSBs.

Table A.6.6.8.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| CSI-RS configuration | 1 |  | CSI-RS.1.3 FDD |
|  | 2 |  | CSI-RS.1.3TDD |
|  | 3 |  | CSI-RS.2.3TDD |
| OCNG Patterns | 1~3 |  | OP.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| DRX configuration | 1~3 |  | DRX.3 |
| reportConfigType | 1~3 |  | aperiodic |
| reportQuantity-r16 | 1~3 |  | cri-SINR-r16 |
| Number of reported RS | 1~3 |  | 2 |
| qcl-Info | 1~3 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1~3 | slots | 26 |
| T1 | 1~3 | s | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.6.6.8.2-2: CSI-RS specific test parameters

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Note1 | 1~3 | dBm/15kHz | -94.65 |  |
| Note1 | 1,2 | dBm/SSB SCS | -94.65 |  |
|  | 3 |  | -91.65 |  |
|  | 1~3 | dB | 0 | 3 |
| CSI-RS RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -91.65 |
|  | 3 |  | -91.65 | -88.65 |
| Io Note2 | 1,2 | dBm/9.36 MHz | -63.69 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -55.84 |
|  | 1~3 | dB | 0 | 3 |
| Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.6.6.8.3	Test Requirements

After 80ms from the beginning of the test, the UE shall send L1-SINR report at slot 26 from the reception of DCI triggering the L1-SINR measurement. The L1-SINR report shall include the results for both CSI-RS#0 and CSI-RS#1 while meeting the absolute accuracy requirement in clause 10.1.27.1.1 and relative accuracy requirement in clause 10.1.27.1.2.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.8.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used

##### A.6.6.8.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-SINR measurement. This test will partly verify the L1-SINR measurement requirements in clause 9.8.4.2, with the testing configurations for NR cells in Table A.6.6.8.2.1-1.

Table A.6.6.8.2.1-1: Applicable NR configurations for FR1 L1-SINR measurement test with SSB based CMR and CSI-RS based IMR

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.8.2.2	Test parameters

There is one cells in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.6.6.8.2.2-1 and Table A.6.6.8.2.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-SINR measurement on the SSBs and the associated CSI-RS resources, and report periodically. The test consists of two successive time periods, with time duration of T1 and T2 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM and BFD measurements based on the SSBs, and UE is configured to perform L1-SINR measurement based on the SSBs as CMR and the CSI-RS resources as IMR.

Table A.6.6.8.2.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | c |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| CSI-RS configuration | 1 |  | CSI-RS 1.1A FDD |
|  | 2 |  | CSI-RS 1.1A TDD |
|  | 3 |  | CSI-RS 2.1A TDD |
| OCNG Patterns | 1~3 |  | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | off |
| reportConfigType | 1~3 |  | periodic |
| reportQuantity-r16 | 1~3 |  | ssb-Index-SINR-r16 |
| Number of reported RS | 1~3 |  | 2 |
| L1-SINR reporting period | 1~3 | slot | 80 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 1 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.6.6.8.2.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -91.65 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -88.65 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.84 |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.8.2.2-3: CSI-RS specific test parameters

| Parameter | Config | Unit | CSI-RS#0 |  | CSI-RS#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/CSI-RS SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| CSI-RS RSRP Note3 | 1,2 | dBm/CSI-RS SCS | -94.65 | -94.65 | -Infinity | -91.65 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -88.65 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.84 |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.8.2.3	Test Requirements

The UE shall send L1-SINR report every 80 slots. No later than 640ms plus 80 slots from the beginning of time period T2, UE shall send L1-SINR report including results of both SSB#0+CSI-RS#0 and SSB#1+CSI-RS#1 while meeting the accuracy requirement in clause 10.1.27.2. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.8.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used

##### A.6.6.8.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-SINR measurement. This test will partly verify the L1-SINR measurement requirements with CSI-RS based CMR and dedicated IMR cofigured in clause 9.8.4.3, with the testing configurations for NR cells in Table A.6.6.8.3.1-1.

Table A.6.6.8.3.1-1: Applicable NR configurations for FR1 L1-SINR test with CMR and dedicated IMR

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.8.3.2	Test parameters

There is one cells in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.6.6.8.3.2-1 and Table A.6.6.8.3.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-SINR measurement on the configured CSI-RS as CMR and an associated CSI-IM as IMR, and report aperiodically. The test consists of a single time period T1, during which the UE is triggered via DCI to report L1-SINR on aperiodic CSI-RS resources. UE is also configured to measure L1-SINR based on SSB. After 80ms from the beginning of the test, the DCI trigger comes in slot n (1 Config 1,2 and 8 for Config 3) of a frame and UE provides the report back based on the reporting configuration as defined in Table A.6.6.8.3.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM and BFD based on the SSBs, and UE is configured to perform L1-SINR measurement based on the CSI-RS as CMR and the CSI-IM as IMR.

Table A.6.6.8.3.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| CSI-RS configuration | 1 |  | CSI-RS 1.3 FDD |
|  | 2 |  | CSI-RS 1.3 TDD |
|  | 3 |  | CSI-RS 2.3 TDD |
| CSI-IM configuration | 1 |  | CSI-IM.1.2 FDD |
|  | 2 |  | CSI-IM.1.2 TDD |
|  | 3 |  | CSI-IM.2.2 TDD |
| OCNG Patterns | 1~3 |  | OP.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| DRX configuration | 1~3 |  | Off |
| reportConfigType | 1~3 |  | aperiodic |
| reportQuantity-r16 | 1~3 |  | cri-SINR-r16 |
| Number of reported RS | 1~3 |  | 2 |
| qcl-Info | 1~3 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1~3 | slots | 26 |
| T1 | 1~3 | s | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

A.6.6.8.3.2-2: CSI-RS specific test parameters

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Note1 | 1~3 | dBm/15kHz | -94.65 |  |
| Note1 | 1,2 | dBm/SSB SCS | -94.65 |  |
|  | 3 |  | -91.65 |  |
|  | 1~3 | dB | 0 | 3 |
| CSI-RS RSRP Note2 | 1,2 | dBm/SSB SCS | -94.65 | -91.65 |
|  | 3 |  | -91.65 | -88.65 |
| Io Note2 | 1,2 | dBm/9.36 MHz | -63.69 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -55.84 |
|  | 1~3 | dB | 0 | 3 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.6.6.8.3.3	Test Requirements

After 80ms from the beginning of the test, the UE shall send L1-SINR report at slot 26 from the reception of DCI triggering the L1-SINR measurement. The L1-SINR report shall include the results for both CSI-RS#0 as CMR + CSI-IM#0 as IMR and CSI-RS#1 as CMR + CSI-IM#1 as IMR while meeting the accuracy requirement in clause 10.1.27.3.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.6.6.9	Idle Mode CA/DC Measurements

#### A.6.6.9.1	SA Idle mode CA/DC measurement for FR1

##### A.6.6.9.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE performs the required measurements on the serving cell and the configured inter-frequency carrier for idle mode measurement reporting after the UE has entered Idle mode. This test will partly verify the Idle mode CA/DC measurements requirements in clause 4.4.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2.  The test parameters are given in Tables A.6.6.9.1.1-1, A.6.6.9.1.1-2, A.6.6.9.1.1-3 and A.6.6.9.1.1-4.

The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. During T1, the UE is connected to cell 1 only and shall not have any timing information of cell 2. UE is configured with early measurement reporting with channel 2. Beam level reporting for early measurements is not configured. The connection is released at the end of T1. T2 starts when the connection is released. During the time periods T2 UE is in Idle mode. At T3 the UE is paged for connection setup and requested by the network to send idle mode measurements.

Table A.6.6.9.1.1-1: supported test configuration

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.9.1.1-2: General test parameters for SA Idle mode CA/DC measurement for FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 | NR cell 2 is on NR RF channel number 2. |
| SMTC-SSB parameters |  | Config 1 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 2 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.2 FR1 | As specified in clause A.3.10.1 |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| PRACH configuration index |  | Config 1,2,3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX in connected mode |  | Config 1,2,3 | OFF | DRX is not used |
| DRX in idle mode | s | Config 1,2,3 | 0.32 | The value shall be used for all cells in the test. |
| T331 | s |  | 300 |  |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 10 |  |
| T2 | s | Config 1,2,3 | 11.52 |  |
| T3 | s | Config 1,2,3 | 10 |  |

Table A.6.6.9.1.1-3: Cell specific test parameters for connected mode for SA Idle mode CA/DC measurement for FR1

| Parameter |  | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T1 | T2 |  | T3 |  |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  | 2 |  |  |  |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  | NA |  |  |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  | NA |  |  |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  | NA |  |  |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  | NA |  |  |  |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  | NA |  |  |  |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  | NA |  |  |  |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  | NA |  |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  | OP.1 |  |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  | SR2.1 TDD |  |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  | CR2.1 TDD |  |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |  |  |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |  |  |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  | SSB.6 FR1 |  |  |  |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  |  | SMTC.5 |  |  |  |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  | SMTC.4 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  |  | -98 |  |  |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  |  | -98 |  |  |  |  |
|  |  |  | Config 3 | -95 |  |  | -95 |  |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -91 | -91 | -91 | -infinity |  | -98 |  | -98 |
|  |  |  | Config 3 | -88 | -88 | -88 | -infinity |  | -95 |  | -95 |
|  |  | dB | Config 1,2,3,4,5,6 | 7 | 7 | 7 | - infinity |  | 0 |  | 0 |
|  |  | dB | Config 1,2,3 | 7 | 7 | 7 | infinity |  | 0 |  | 0 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -62.26 | -62.26 | -62.26 | -70.5 |  | -67.04 |  | -67.04 |
|  |  | dBm/38.16MHz | Config 3 | -56.15 | -56.15 | -56.15 | -63.94 |  | -60.93 |  | -60.93 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  |  | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |  |

Table A.6.6.9.1.1-4: Cell specific test parameters for idle mode for SA Idle mode CA/DC measurement for FR1

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  | 1,2,3 | 1 |  |  | 2 |  |  |
| TDD configuration |  | 1 | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC |  | 1 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
| configuration |  | 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET |  | 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET |  | 1 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -140 |  |  | -140 |  |  |
|  |  | 3 | -137 |  |  | -137 |  |  |
| Pcompensation | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3 | SS-RSRP |  |  | SS-RSRP |  |  |
|  | dB | 1 | 14 | 14 | 14 | -infinity | 12 | 12 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
|  | dB | 1 | 7 | 7 | 7 | -infinity | 0 | 0 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| ÊsIot | dB | 1, 2, 3 | 7 | 7 | 7 | -infinity | 0 | 0 |
| SS-RSRP Note3 | dBm/SCS | 1 | -91 | -91 | -91 | -infinity | -98 | -98 |
|  |  | 2 | -91 | -91 | -91 | -infinity | -98 | -98 |
|  |  | 3 | -88 | -88 | -88 | -infinity | -95 | -95 |
| Io | dBm/9.36 MHz | 1 | -62.26 | -62.26 | -62.26 | -70.5 | -67.04 | -67.04 |
|  | dBm/9.36 MHz | 2 | -62.26 | -62.26 | -62.26 | -70.5 | -67.04 | -67.04 |
|  | dBm/38.16 MHz | 3 | -56.15 | -56.15 | -56.15 | -63.94 | -60.93 | -60.93 |
| Treselection | s | 1, 2, 3 | 0 | 0 | 0 | 0 | 0 | 0 |
| SnonintrasearchP | dB | 1, 2, 3 | Not sent |  |  | Not sent |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

##### A.6.6.9.1.2	Test Requirements

The UE behaviour during time durations T2 and T3 shall be as follows:

During the time period T2 the UE is in Idle mode and the signal level of cell 2 is changed. The UE shall not perform reselection. The UE shall perform Idle Mode CA measurement according to Section 4.4.

At the start of T3 the UE is paged for connection setup. During the connection setup the UE is requested to transmit early measurement report for cell 2. The UE shall send early measurement report to the PCell.

After receiving the requested early measurement report, the test equipment verifies the accuracy of measurement reported for Cell 2 meets the requirements in Section 10.1.4B for SS-RSRP and in Section 10.1.8B for SS-RSRQ and test ends.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.6.6.10	CSI-RS based intra-frequency Measurements

#### A.6.6.10.1	SA event triggered reporting tests without gap under non-DRX

##### A.6.6.10.1.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA CSI-RS based L3 intra-frequency requirements in clauses 9.10.2.

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell and neighbour cell are given in Table A.6.6.10.1.1-1 and A.6.6.10.1.1-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

Table A.6.6.10.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.10.1.1-2: General test parameters for SA intra-frequency event triggered reporting without gap for FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified and measured. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CSI-RS configuration for RRM |  | 1 | CSI-RS.RRM.FR1.1 FDD |  |
|  |  | 2 | CSI-RS.RRM.FR1.1 TDD |  |
|  |  | 3 | CSI-RS.RRM.FR1.2 TDD |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells | s | 1 | 4.7 | Asynchronous cells. The timing of Cell 2 is CP later than the timing of Cell 1. |
|  |  | 2 | 4.7 | Synchronous cells |
|  |  | 3 | 2.35 | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 1 |  |

Table A.6.6.10.1.1-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for FR1

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | TN/A |  | TN/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| CSI-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | --58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.10.1.2	Test Requirements

In this test, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE is required to read the neighbour cell SSB index and report the acquired SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.6.6.11	CSI-RS based inter-frequency Measurements

#### A.6.6.11.1	 SA event triggered reporting tests with gap under DRX

##### A.6.6.11.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA CSI-RS based L3 inter-frequency measurement requirements in clause 9.10.3.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2. The test parameters are given in Tables A.6.6.11.1.1-1, A.6.6.11.1.1-2 and A.6.6.11.1.1-3.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table A.6.6.11.1.1-2 is provided for UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #4 as defined in Table A.6.6.11.1.1-2 is provided for UE that supports per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

UE needs to be provided at least once every 500 ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

Table A.6.6.11.1.1-1: SA event triggered reporting tests for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.11.1.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 | 4 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 39 | 19 |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.5 | DRX.5 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells | s | Config 1 | 4.7 |  | Asynchronous cells. The timing of Cell 2 is CP later than the timing of Cell 1. |
|  |  | Config 2 | 4.7 |  | Synchronous cells. |
|  |  | Config 3 | 2.35 |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 10 | 10 |  |

Table A.6.6.11.1.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 with SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| CSI-RS configuration for RRM |  |  | Config 1 | CSI-RS.RRM.FR1.1 FDD |  | CSI-RS.RRM.FR1.1 FDD |  |
|  |  |  | Config 2 | CSI-RS.RRM.FR1.1 TDD |  | CSI-RS.RRM.FR1.1 TDD |  |
|  |  |  | Config 3 | CSI-RS.RRM.FR1.2 TDD |  | CSI-RS.RRM.FR1.2 TDD |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| CSI-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

##### A.6.6.11.1.2	Test Requirements

In test 1 with per-UE gap and test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 9280 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2 UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.6.6.12	RSTD measurements

#### A.6.6.12.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA

##### A.6.6.12.1.1	Test Purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the requirements specified in Clause 9.9.2 in an environment with AWGN propagation conditions in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations are specified in Table A.6.6.12.1.1-1.

Table A.6.6.12.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All 3 cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. During time duration T1, the UE shall not have any timing information of Cell 2 and Cell 3. All three cells transmit PRS during T2.

Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The NR-DL-TDOA-ProvideAssistanceData and nr-DL-TDOA-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-TDOA assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources.

The UE is configured with measurement gap pattern ID # 24 or #0 before T2.

The general test parameters are listed in Table A.6.6.12.1.1-2, and cell specific test parameters are listed in Table A.6.6.12.1.1-3.

Table A.6.6.12.1.1-2: General test parameters for RSTD measurement reporting delay

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  |  | Cell 1 | Reference cell is the cell in the DL-TDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 38.215 [4] and TS 37.355 [34]. The reference cell is the PCell in this test case. |
| Neighbor cells |  |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at the first and second places in the neighbour cell list in the DL-TDOA assistance data. |
| SSB configuration | Config 1 |  | SSB.1 FR1 |  |
|  | Config 2 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB.2 FR1 |  |
| SMTC configuration | Config 1 |  | SMTC.2 |  |
|  | Config 2 |  | SMTC.1 |  |
|  | Config 3 |  | SMTC.1 |  |
| PDSCH RMC configuration | Config 1 |  | SR.1.1 FDD |  |
|  | Config 2 |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration | Config 1 |  | CR.1.1 FDD | As specified in clause A.3.1.2.1 |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration | Config 1 |  | CCR.1.1 FDD |  |
|  | Config 2 |  | CCR.1.1 TDD |  |
|  | Config 3 |  | CCR.2.1 TDD |  |
| Initial BWP configuration | Config 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration | Config 1,2,3 |  | DLBWP.1.1 |  |
| Active UL BWP configuration | Config 1,2,3 |  | ULBWP.1.1 |  |
| PRS Configuration | Config 1 |  | PRS.1.1 FR1 | As specified in clause A.3.31 |
|  | Config 2 |  | PRS.1.1 FR1 |  |
|  | Config 3 |  | PRS.2.1 FR1 |  |
| Physical cell ID PCI |  |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| Measurement gap |  |  | GP#24 or GP#0 | GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured |
| Radio frame receive time offset between the cells at the UE antenna connector |  | s | Cell 2 to Cell 1: 0 Cell 3 to Cell 1: 3 | PRS are transmitted from synchronous cells |
| Expected RSTD |  | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355 [34] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells |  | s | 5 | The corresponding parameter in the DL-TDOA assistance ta specified in TS 37.355 [34] is the expectedRSTD-Uncertainty index |
| Number of cells provided in DL-TDOA assistance data |  |  | 16 | Including the reference cell |
| PRS muting info |  |  | Cell 1: ‘10’ Cell 2: ‘01’ Cell 3: ‘10’ | Correponds to prs-MutingInfo defined in TS 37.355 [34] |
| PRS resource RE offset |  |  | Cell 1: 0 Cell 2: 0 Cell 3: 1 | Cell 1 and Cell 3 are configured with different resource offsets |
| T1 |  | s | 3 | The length of the time interval from the beginning of each test |
| T2 |  | s | 1.28 | The length of the time interval that follows immediately after time interval T1 |

Table A.6.6.12.1.1-3: Cell-specific test parameters for RSTD measurement reporting delay during T1

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Positiong frequency layer |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | N/A | N/A |
| EPRE ratio of PSS to SSS |  | dB | 0 | N/A | N/A |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note 3 | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 | dBm/SCS | -98 |  |  |
|  | Config 3 | dBm/SCS | -95 |  |  |
| PRS |  | dB | -Infinity | -Infinity | -Infinity |
| SSB |  | dB | 10 | -Infinity | -Infinity |
| Io Note 4 | Config 1 | dBm/ 9.36MHz | -59.63 | -59.63 | -59.63 |
|  | Config 2 | dBm/ 9.36MHz | -59.63 | -59.63 | -59.63 |
|  | Config 3 | dBm/ 38.16MHz | -53.54 | -53.54 | -53.54 |
| SSB RP Note4 | Config 1 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 2 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 3 | dBm/SCS | -85 | -Infinity | -Infinity |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the slots with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |

Table A.6.6.12.1.1-4: Cell-specific test parameters for RSTD measurement reporting delay during T2

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T2 | T2 | T2 |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | OP.1 | OP.1 |
| PRACH configuration |  |  | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note 3 | Config 1 | dBm/SCS | -98 | -98 | -98 |
|  | Config 2 | dBm/SCS | -98 | -98 | -98 |
|  | Config 3 | dBm/SCS | -95 | -95 | -95 |
| PRS | Config 1 | dB | -5.45 | -11.67 | -11.67 |
|  | Config 2 | dB | -5.45 | -11.67 | -11.67 |
|  | Config 3 | dB | -5.45 | -11.67 | -11.67 |
| SSB | Config 1~3 | dB | 10 | 3 | 3 |
| Io Note 4 | Config 1 | dBm/ 9.36MHz | -68.52 | -68.52 | -68.52 |
|  | Config 2 | dBm/ 9.36MHz | -68.52 | -68.52 | -68.52 |
|  | Config 3 | dBm/ 38.16MHz | -62.43 | -62.43 | -62.43 |
| SSB RP Note4 | Config 1 | dBm/SCS | -88 | -95 | -95 |
|  | Config 2 | dBm/SCS | -88 | -95 | -95 |
|  | Config 3 | dBm/SCS | -85 | -92 | -92 |
| PRS |  | dB | -6.00 | -12.98 | -12.98 |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the slots with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. The Io is calculated based only on the symbols in which PRS is transmitted. |  |  |  |  |  |

##### A.6.6.12.1.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in Clause 9.9.2.5.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the DL-TDOA assistance data, Cell 1, within the time duration specified in section 9.9.2.5 starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 10.1.23.3, i.e., between RSTD_0000000 and RSTD1970049

#### A.6.6.12.2	NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR1 SA

##### A.6.6.12.2.1	Test Purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the requirements specified in Clause 9.9.2 in an environment with AWGN propagation conditions in FR1 in standalone scenario when dual positioning frequency layers are configured.

The supported test configurations are specified in Table A.6.6.12.2.1-1.

Table A.6.6.12.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. Cell 3 is on a different RF channel with Cell 1 and Cell 2.

The test consists of two consecutive time intervals, with duration of T1 and T2. During time duration T1, the UE shall not have any timing information of Cell 2 and Cell 3. All three cells transmit PRS during T2.

Note:	The information on when PRS is muted is conveyed to the UE using PRS muting information.

The NR-DL-TDOA-ProvideAssistanceData and nr-DL-TDOA-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-TDOA assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources.

The UE is configured with measurement gap pattern ID # 24 or #0 before T2.

The general test parameters are listed in Table A.6.6.12.2.1-2, and cell specific test parameters are listed in Table A.6.6.12.2.1-3.

Table A.6.6.12.2.1-2: General test parameters for RSTD measurement reporting delay

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  |  | Cell 1 | Reference cell is the cell in the DL-TDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 38.215 [4] and TS 37.355 [34]. The reference cell is the PCell in this test case. |
| Neighbor cells |  |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at the first and second places in the neighbour cell list in the DL-TDOA assistance data. |
| SSB configuration | Config 1 |  | SSB.1 FR1 |  |
|  | Config 2 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB.2 FR1 |  |
| SMTC configuration | Config 1 |  | SMTC.2 |  |
|  | Config 2 |  | SMTC.1 |  |
|  | Config 3 |  | SMTC.1 |  |
| PDSCH RMC configuration | Config 1 |  | SR.1.1 FDD |  |
|  | Config 2 |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration | Config 1 |  | CR.1.1 FDD | As specified in clause A.3.1.2.1 |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration | Config 1 |  | CCR.1.1 FDD |  |
|  | Config 2 |  | CCR.1.1 TDD |  |
|  | Config 3 |  | CCR.2.1 TDD |  |
| Initial BWP configuration | Config 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration | Config 1,2,3 |  | DLBWP.1.1 |  |
| Active UL BWP configuration | Config 1,2,3 |  | ULBWP.1.1 |  |
| PRS Configuration | Config 1 |  | PRS.1.1 FR1 | As specified in clause A.3.31 |
|  | Config 2 |  | PRS.1.1 FR1 |  |
|  | Config 3 |  | PRS.2.1 FR1 |  |
| Physical cell ID PCI |  |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| Measurement gap |  |  | GP#24 or GP#0 | GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured |
| Radio frame receive time offset between the cells at the UE antenna connector |  | s | Cell 2 to Cell 1: 0 Cell 3 to Cell 1: 3 | PRS are transmitted from synchronous cells |
| Expected RSTD |  | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355 [34] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells |  | s | 5 | The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355[34] is the expectedRSTD-Uncertainty index |
| Number of cells provided in DL-TDOA assistance data |  |  | 16 | Including the reference cell |
| PRS muting info |  |  | Cell 1: ‘10’ Cell 2: ‘01’ Cell 3: ‘10’ | Correponds to prs-MutingInfo defined in TS 37.355 [34] |
| PRS resource RE offset |  |  | Cell 1: 0 Cell 2: 0 Cell 3: 1 | Cell 1 and Cell 3 are configured with different resource offsets |
| T1 |  | s | 3 | The length of the time interval from the beginning of each test |
| T2 |  | s | 1.28 | The length of the time interval that follows immediately after time interval T1 |

Table A.6.6.12.2.1-3: Cell-specific test parameters for RSTD measurement reporting delay during T1

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 | 2 |
| Positiong frequency layer |  |  | 1 | 1 | 2 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | N/A | N/A |
| EPRE ratio of PSS to SSS |  | dB | 0 | N/A | N/A |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note 3 | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 | dBm/SCS | -98 |  |  |
|  | Config 3 | dBm/SCS | -95 |  |  |
| PRS |  | dB | -Infinity | -Infinity | -Infinity |
| SSB |  | dB | 10 | -Infinity | -Infinity |
| Io Note 4 | Config 1 | dBm/ 9.36MHz | -59.63 | -59.63 | -70.05 |
|  | Config 2 | dBm/ 9.36MHz | -59.63 | -59.63 | -70.05 |
|  | Config 3 | dBm/ 38.16MHz | -53.54 | -53.54 | -63.96 |
| SSB RP Note4 | Config 1 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 2 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 3 | dBm/SCS | -85 | -Infinity | -Infinity |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the slots with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |

Table A.6.6.12.2.1-4: Cell-specific test parameters for RSTD measurement reporting delay during T2

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T2 | T2 | T2 |
| NR RF Channel Number |  |  | 1 | 1 | 2 |
| Positiong frequency layer |  |  | 1 | 1 | 2 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | OP.1 | OP.1 |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |
| PRACH configuration |  |  | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 |
| Note 3 | Config 1 | dBm/SCS | -98 | -98 | -98 |
|  | Config 2 | dBm/SCS | -98 | -98 | -98 |
|  | Config 3 | dBm/SCS | -95 | -95 | -95 |
| PRS | Config 1 | dB | -5.45 | -11.67 | -11.67 |
|  | Config 2 | dB | -5.45 | -11.67 | -11.67 |
|  | Config 3 | dB | -5.45 | -11.67 | -11.67 |
| Io Note 4 | Config 1 | dBm/ 9.36MHz | -68.73 | -68.73 | -69.76 |
|  | Config 2 | dBm/ 96.48MHz | -68.73 | -68.73 | -69.76 |
|  | Config 3 | dBm/ 38.16MHz | -62.64 | -62.64 | -63.67 |
| PRS |  | dB | -6 | -13 | -13 |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the slots with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. The Io is calculated based only on the symbols in which PRS is transmitted. |  |  |  |  |  |

##### A.6.6.12.2.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in Clause 9.9.2.5.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the DL-TDOA assistance data, Cell 1, within the time duration specified in section 9.9.1.5 starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 10.1.23.3, i.e., between RSTD_0000000 and RSTD_1970049.

#### A.6.6.12.3	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA

##### A.6.6.12.3.1	Test Purpose and Environment

The purpose of the test is to verify that the RSTD measurement with the reduced samples meets the requirements specified in Clause 9.9.2 in an environment with AWGN propagation conditions in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations are specified in Table A.6.6.12.3.1-1.

Table A.6.6.12.3.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All 3 cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. During time duration T1, the UE shall not have any timing information of Cell 2 and Cell 3. All three cells transmit PRS during T2.

Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The NR-DL-TDOA-ProvideAssistanceData and nr-DL-TDOA-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-TDOA assistance data and location information request. UE can support [M-sample measurements], and the LMF indicates the UE to perform positioning measurements with reduced number of samples

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources.

The UE is configured with measurement gap pattern ID # 24 or #0 before T2.

The general test parameters are listed in Table A.6.6.12.3.1-2, and cell specific test parameters are listed in Table A.6.6.12.3.1-3.

Table A.6.6.12.3.1-2: General test parameters for RSTD measurement reporting delay

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  |  | Cell 1 | Reference cell is the cell in the DL-TDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 38.215 [4] and TS 37.355 [34]. The reference cell is the PCell in this test case. |
| Neighbor cells |  |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at the first and second places in the neighbour cell list in the DL-TDOA assistance data. |
| SSB configuration | Config 1 |  | SSB.1 FR1 |  |
|  | Config 2 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB.2 FR1 |  |
| SMTC configuration | Config 1 |  | SMTC.2 |  |
|  | Config 2 |  | SMTC.1 |  |
|  | Config 3 |  | SMTC.1 |  |
| PDSCH RMC configuration | Config 1 |  | SR.1.1 FDD |  |
|  | Config 2 |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration | Config 1 |  | CR.1.1 FDD | As specified in clause A.3.1.2.1 |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration | Config 1 |  | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Initial BWP configuration | Config 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration | Config 1,2,3 |  | DLBWP.1.1 |  |
| Active UL BWP configuration | Config 1,2,3 |  | ULBWP.1.1 |  |
| PRS Configuration | Config 1 |  | PRS.1.2 FR1 | As specified in clause A.3.31 |
|  | Config 2 |  | PRS.1.2 FR1 |  |
|  | Config 3 |  | PRS.2.1 FR1 |  |
| Physical cell ID PCI |  |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| Measurement gap |  |  | GP#24 or GP#0 | GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured |
| Radio frame receive time offset between the cells at the UE antenna connector |  | s | Cell 2 to Cell 1: 0 Cell 3 to Cell 1: 3 | PRS are transmitted from synchronous cells |
| Expected RSTD |  | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355 [34] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells |  | s | 5 | The corresponding parameter in the DL-TDOA assistance ta specified in TS 37.355 [34] is the expectedRSTD-Uncertainty index |
| Number of cells provided in DL-TDOA assistance data |  |  | 16 | Including the reference cell |
| PRS muting info |  |  | Cell 1: ‘10’ Cell 2: ‘01’ Cell 3: ‘10’ | Correponds to prs-MutingInfo defined in TS 37.355 [34] |
| PRS resource RE offset |  |  | Cell 1: 0 Cell 2: 0 Cell 3: 1 | Cell 1 and Cell 3 are configured with different resource offsets |
| T1 |  | s | 3 | The length of the time interval from the beginning of each test |
| T2 |  | s | [1.28] | The length of the time interval that follows immediately after time interval T1 |

Table A.6.6.12.3.1-3: Cell-specific test parameters for RSTD measurement reporting delay during T1

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Positiong frequency layer |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | N/A | N/A |
| Note 3 | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 | dBm/SCS | -98 |  |  |
|  | Config 3 | dBm/SCS | -95 |  |  |
| PRS |  | dB | -Infinity | -Infinity | -Infinity |
| SSB |  | dB | 10 | -Infinity | -Infinity |
| Io Note 4 | Config 1 | dBm/ 9.36MHz | -68.63 | -70.05 | -70.05 |
|  | Config 2 | dBm/ 9.36MHz | -68.63 | -70.05 | -70.05 |
|  | Config 3 | dBm/ 38.16MHz | -63.20 | -63.96 | -63.96 |
| SSB RP Note4 | Config 1 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 2 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 3 | dBm/SCS | -88 | -Infinity | -Infinity |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |

Table A.6.6.12.3.1-4: Cell-specific test parameters for RSTD measurement reporting delay during T2

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T2 | T2 | T2 |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | OP.1 | OP.1 |
| PRACH configuration |  |  | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 |
| Note 3 | Config 1 | dBm/SCS | -98 | -98 | -98 |
|  | Config 2 | dBm/SCS | -98 | -98 | -98 |
|  | Config 3 | dBm/SCS | -95 | -95 | -95 |
| PRS | Config 1 | dB | -2 | -5.45 | -5.45 |
|  | Config 2 | dB | -2 | -5.45 | -5.45 |
|  | Config 3 | dB | -2 | -5.45 | -5.45 |
| Io Note 4 | Config 1 | dBm/ 9.36MHz | -67.67 | -69.59 | -69.59 |
|  | Config 2 | dBm/ 96.48MHz | -67.67 | -69.59 | -69.59 |
|  | Config 3 | dBm/ 38.16MHz | -61.57 | -63.72 | -63.72 |
| PRS |  | dB | -3 | -6 | -6 |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |  |

##### A.6.6.12.3.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in Clause 9.9.2.5

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the DL-TDOA assistance data, Cell 1, within the time duration X1 specified in section 9.9.1.5 starting from the beginning of time interval T2, where X1 is 320ms.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 10.1.23.3, i.e., between RSTD_0000000 and RSTD1970049.

#### A.6.6.12.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA without measurement gap

##### A.6.6.12.4.1	Test Purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the gapless RSTD measurement period requirement specified in Clause 9.9.2.7 in an environment with AWGN propagation conditions in FR1 in standalone scenario when single positioning frequency layer is configured. Reporting delay test for gapless PRS measurement is conducted assuming that the PRS has higher priority, i.e., state 1, than all other DL signals/channels and is transmitted within active DL BWP of UE. Two sub-tests are defined, sub-test 1 is for Nsample = 4 and sub-test 2 is for Nsample = 1. For sub-test 2 LMF indicates UE to perform PRS measurement with Nsample = 1. The cell specific parameters for sub-test 1 and sub-test 2 are defined in Table A.6.6.12.4.1-4.

The supported test configurations are specified in Table A.6.6.12.4.1-1.

Table A.6.6.12.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All 3 cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. During time duration T1, the UE shall not have any timing information of Cell 2 and Cell 3. All three cells transmit PRS during T2.

Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The NR-DL-TDOA-ProvideAssistanceData and nr-DL-TDOA-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-TDOA assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of the first PPW instance containing the PRS resources.

The UE is configured with PPW before start of T2.

The general test parameters are listed in Table A.6.6.12.4.1-2, and cell specific test parameters are listed in Table A.6.6.12.4.1-3.

Table A.6.6.12.4.1-2: General test parameters for RSTD measurement reporting delay

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  |  | Cell 1 | Reference cell is the cell in the DL-TDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 38.215 [4] and TS 37.355 [34]. The reference cell is the PCell in this test case. |
| Neighbor cells |  |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at the first and second places in the neighbour cell list in the DL-TDOA assistance data. |
| BWchannel | Config 1 | MHz | 10: NRB,c = 52 |  |
|  | Config 2 |  | 10: NRB,c = 52 |  |
|  | Config 3 |  | 40: NRB,c = 106 |  |
| SSB configuration | Config 1 |  | SSB.1 FR1 |  |
|  | Config 2 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB.2 FR1 |  |
| SMTC configuration | Config 1 |  | SMTC.2 |  |
|  | Config 2 |  | SMTC.1 |  |
|  | Config 3 |  | SMTC.1 |  |
| PDSCH RMC configuration | Config 1 |  | SR.1.1 FDD |  |
|  | Config 2 |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration | Config 1 |  | CR.1.1 FDD | As specified in clause A.3.1.2.1 |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration | Config 1 |  | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Initial BWP configuration | Config 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration | Config 1,2,3 |  | DLBWP.1.1 | Cell 2 and cell 3 also transmit PRS on active DL BWP configured to the UE |
| Active UL BWP configuration | Config 1,2,3 |  | ULBWP.1.1 |  |
| PRS Configuration | Config 1 |  | PRS.1.4 FR1 | As specified in clause A.3.31 |
|  | Config 2 |  | PRS.1.4 FR1 |  |
|  | Config 3 |  | PRS.2.4 FR1 |  |
| PRS BW | Config 1,2,3 |  | 48 PRBs | Applicable to sub-test 2. |
| Physical cell ID PCI |  |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| PPW configuration |  |  | Table A.3.X-1: Reference PPW configuration | As defined in A.3.X |
| Time offset between serving and neighbour cells |  | s | - set to the UE reported capability for receive time difference threshold if the UE reported value is < 3µs - 3µs otherwise | PRS are transmitted from synchronous cells |
| Expected RSTD |  | s | 0 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355 [34] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells |  | s | Same as time offset | The corresponding parameter in the DL-TDOA assistance ta specified in TS 37.355 [34] is the expectedRSTD-Uncertainty index |
| Number of cells provided in DL-TDOA assistance data |  |  | 16 | Including the reference cell |
| PRS muting info |  |  | Cell 1: ‘10’ Cell 2: ‘01’ Cell 3: ‘10’ | Correponds to prs-MutingInfo defined in TS 37.355 [34] |
| PRS resource RE offset |  |  | Cell 1: 0 Cell 2: 0 Cell 3: 1 | Cell 1 and Cell 3 are configured with different resource offsets |
| T1 |  | s | 3 | The length of the time interval from the beginning of each test |
| T2 |  | ms | 10 | The length of the time interval that follows after time interval T1 and processing time of assistance data. In this test UE is configured to measure single PFL within the configured PPW. |

Table A.6.6.12.4.1-3: Cell-specific test parameters for RSTD measurement reporting delay during T1

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Positiong frequency layer |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | N/A | N/A |
| Note 3 | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 | dBm/SCS | -98 |  |  |
|  | Config 3 | dBm/SCS | -95 |  |  |
| PRS |  | dB | -Infinity | -Infinity | -Infinity |
| SSB |  | dB | 10 | -Infinity | -Infinity |
| Io Note 4 | Config 1 | dBm/ 9.36MHz | -68.63 | -70.05 | -70.05 |
|  | Config 2 | dBm/ 9.36MHz | -68.63 | -70.05 | -70.05 |
|  | Config 3 | dBm/ 38.16MHz | -63.20 | -63.96 | -63.96 |
| SSB RP Note4 | Config 1 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 2 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 3 | dBm/SCS | -88 | -Infinity | -Infinity |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |

Table A.6.6.12.4.1-4: Cell-specific test parameters for RSTD measurement reporting delay during T2

| Parameter |  |  | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Sub-test 1 | Sub-test 2 | Sub-test 1 | Sub-test 2 | Sub-test 1 | Sub-test 2 |
| NR RF Channel Number |  |  |  | 1 |  | 1 |  | 1 |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| OCNG patterns defined in A.3.2.1 |  |  |  | OP.1 |  | OP.1 |  | OP.1 |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  | FR1 PRACH configuration 1 |  | FR1 PRACH configuration 1 |  |
| Note 3 |  | Config 1,2 | dBm/SCS | -98 |  | -98 |  | -98 |  |
|  |  | Config 3 | dBm/SCS | -95 |  | -95 |  | -95 |  |
| PRS |  | Config 1,2,3 | dB | -6 | -3 | -13 | 5 | -13 | 5 |
| Io Note 4 |  | Config 1,2 | dBm/ 9.36MHz | -69.59 | -69.59 | -69.93 | -67.9 | -69.93 | -67.9 |
|  |  | Config 3 | dBm/ 38.16MHz | -63.72 |  | -63.89 |  | -63.89 |  |
| PRS | Config 1,2,3 |  | dB | -6 | -3 | -13 | 5 | -13 | 5 |
| PRS-RSRPNote 4 | Config 1,2 |  | dBm/SCS | -104 | -101 | -111 | -93 | -111 | -93 |
|  | Config 3 |  |  | -101 | -91 | -108 | -90 | -108 | -90 |
| SS-RSRPNote 4 | Config 1,2 |  | dBm/SCS | -88 |  | -88 |  | -88 |  |
|  | Config 3 |  |  | -85 |  | -85 |  | -85 |  |
| Propagation Condition |  |  |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |

##### A.6.6.12.4.2	Test Requirements

The RSTD measurement time fulfils the gapless RSTD measurement period requirements specified in Clause 9.9.2.7.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the DL-TDOA assistance data, Cell 1, within the time duration specified in section 9.9.2.7 starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 10.1.23.3, i.e., between RSTD_0000000 and RSTD1970049.

#### A.6.6.12.5	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_CONNECTED state with Rx TEG

##### A.6.6.12.5.1	Test Purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the Rx TEG based measurement period requirements specified in Clause 9.9.2.5 in an environment with AWGN propagation conditions in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations are specified in Table A.6.6.12.5.1-1.

Table A.6.6.12.5.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All 3 cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. During time duration T1, the UE shall not have any timing information of Cell 2 and Cell 3. All three cells transmit PRS during T2.

Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The NR-DL-TDOA-ProvideAssistanceData and nr-DL-TDOA-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-TDOA assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources.

The UE is configured with measurement gap pattern ID # 24 or #0 before T2.

The test applies to the UE supporting Rx TEG indicated via NR-UE-TEG-Capability and is requested to provide the Rx TEG in the test via nr-UE-RxTEG-Request-r17 in NR-TDOA-RequestLocationInformation. In the location request measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 is set to n0. The UE shall perform and optionally report the Rx TEG based RSTD measurements.

The general test parameters are listed in Table A.6.6.12.5.1-2, and cell specific test parameters are listed in Table A.6.6.12.5.1-3.

Table A.6.6.12.5.1-2: General test parameters for RSTD measurement reporting delay

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Reference cell |  |  |  | Cell 1 | Reference cell is the cell in the DL-TDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 38.215 [4] and TS 37.355 [34]. The reference cell is the PCell in this test case. |
| Neighbor cells |  |  |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at the first and second places in the neighbour cell list in the DL-TDOA assistance data. |
| BWchannel | Config 1 |  | MHz | 10: NRB,c = 52 |  |
|  | Config 2 |  |  | 10: NRB,c = 52 |  |
|  | Config 3 |  |  | 40: NRB,c = 106 |  |
| SSB configuration |  | Config 1 |  | SSB.1 FR1 |  |
|  |  | Config 2 |  | SSB.1 FR1 |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |
| SMTC configuration |  | Config 1 |  | SMTC.2 |  |
|  |  | Config 2 |  | SMTC.1 |  |
|  |  | Config 3 |  | SMTC.1 |  |
| PDSCH RMC configuration |  | Config 1 |  | SR.1.1 FDD |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | Config 1 |  | CR.1.1 FDD | As specified in clause A.3.1.2.1 |
|  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | Config 1 |  | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |
| Initial BWP configuration |  | Config 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | Config 1,2,3 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | Config 1,2,3 |  | ULBWP.1.1 |  |
| PRS Configuration |  | Config 1 |  | PRS.1.4 FR1 | As specified in clause A.3.31 |
|  |  | Config 2 |  | PRS.1.4 FR1 |  |
|  |  | Config 3 |  | PRS.2.4 FR1 |  |
| Physical cell ID PCI |  |  |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  |  |  | Normal |  |
| DRX |  |  |  | OFF |  |
| Measurement gap |  |  |  | GP#24 or GP#0 | GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured |
| Radio frame receive time offset between the cells at the UE antenna connector |  |  | s | Cell 2 to Cell 1: 0 Cell 3 to Cell 1: 3 | PRS are transmitted from synchronous cells |
| Expected RSTD |  |  | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355 [34] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells |  |  | s | 5 | The corresponding parameter in the DL-TDOA assistance ta specified in TS 37.355 [34] is the expectedRSTD-Uncertainty index |
| Number of cells provided in DL-TDOA assistance data |  |  |  | 16 | Including the reference cell |
| PRS muting info |  |  |  | Cell 1: ‘10’ Cell 2: ‘01’ Cell 3: ‘10’ | Correponds to prs-MutingInfo defined in TS 37.355 [34] |
| PRS resource RE offset |  |  |  | Cell 1: 0 Cell 2: 0 Cell 3: 1 | Cell 1 and Cell 3 are configured with different resource offsets |
| T1 |  |  | s | 3 | The length of the time interval from the beginning of each test |
| T2 |  |  | s | [1.28*kmultiTEG]Note 1 | The length of the time interval that follows immediately after time interval T1 |
| Note 1: - kmultiTEG=NTEGkTEG,simul if UE is capable of receiving the same DL PRS resource from the same TRP simultaneously from multiple Rx TEGs, where NTEG is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource, and kTEG,simul is the number of Rx TEGs UE can measure simultaneously which is reported via measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul. |  |  |  |  |  |

Table A.6.6.12.5.1-3: Cell-specific test parameters for RSTD measurement reporting delay during T1

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Positiong frequency layer |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | N/A | N/A |
| Note 3 | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 | dBm/SCS | -98 |  |  |
|  | Config 3 | dBm/SCS | -95 |  |  |
| PRS |  | dB | -Infinity | -Infinity | -Infinity |
| SSB |  | dB | 10 | -Infinity | -Infinity |
| Io Note 4 | Config 1 | dBm/ 9.36MHz | -68.63 | -70.05 | -70.05 |
|  | Config 2 | dBm/ 9.36MHz | -68.63 | -70.05 | -70.05 |
|  | Config 3 | dBm/ 38.16MHz | -63.20 | -63.96 | -63.96 |
| SSB RP Note4 | Config 1 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 2 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 3 | dBm/SCS | -88 | -Infinity | -Infinity |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |

Table A.6.6.12.5.1-4: Cell-specific test parameters for RSTD measurement reporting delay during T2

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T2 | T2 | T2 |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | OP.1 | OP.1 |
| PRACH configuration |  |  | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 |
| Note 3 | Config 1 | dBm/SCS | -98 | -98 | -98 |
|  | Config 2 | dBm/SCS | -98 | -98 | -98 |
|  | Config 3 | dBm/SCS | -95 | -95 | -95 |
| PRS | Config 1 | dB | -5.45 | -11.67 | -11.67 |
|  | Config 2 | dB | -5.45 | -11.67 | -11.67 |
|  | Config 3 | dB | -5.45 | -11.67 | -11.67 |
| Io Note 4 | Config 1 | dBm/ 9.36MHz | -69.59 | -69.93 | -69.93 |
|  | Config 2 | dBm/ 96.48MHz | -69.59 | -69.93 | -69.93 |
|  | Config 3 | dBm/ 38.16MHz | -63.72 | -63.89 | -63.89 |
| PRS |  | dB | -6 | -13 | -13 |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |  |

##### A.6.6.12.5.2	Test Requirements

The RSTD measurement time fulfils the Rx TEG based RSTD measurement period requirements specified in Clause 9.9.2.5. The UE shall perform and report the Rx TEG based RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the DL-TDOA assistance data, Cell 1, within the time duration specified in section 9.9.2.5 starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 10.1.23.3, i.e., between RSTD_0000000 and RSTD1970049.

### A.6.6.13 PRS-RSRP measurements

#### A.6.6.13.1	PRS-RSRP reporting delay test case for single positioning frequency layer

##### A.6.6.13.1.1	Test purpose and Environment

The purpose of the test is to verify that the PRS-RSRP measurement meets the delay requirements specified in clause 9.9.3.5 in an environment with AWGN propagation conditions.

The supported test configurations are specified in Table A.6.6.13.1.1-1.

Table A.6.6.13.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR1. The test consists of two consecutive time intervals, with duration of T1 and T2. Both cells transmit PRS during T2.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The general test parameters are listed in Table A.6.6.13.1.1-2, and cell specific test parameters are listed in Table A.6.6.13.1.1-3.

Table A.6.6.13.1.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |
|  |  | 2 | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| Measurement gap |  | 1, 2, 3 | GP#24 or GP#0 Note 1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | NA | OFF |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | 5 |  |
| NOTE 1: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

Table A.6.6.13.1.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 2 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 3 | PRS.2.4 FR1 |  | PRS.2.4 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -3.41 | -Infinity | -11.76 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -101 | -Infinity | -108 |
|  |  | 2 | -Infinity | -101 | -Infinity | -108 |
|  |  | 3 | -Infinity | -98 | -Infinity | -105 |
| Io | dBm/9.36 MHz | 1 | N/A | -68 | N/A | -68 |
|  | dBm/9.36 MHz | 2 |  | -68 |  | -68 |
|  | dBm/38.16 MHz | 3 |  | -61.91 |  | -61.91 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.13.1.2	Test Requirements

The UE shall perform and report the PRS-RSRP measurements for Cell 1 and Cell 2, within the time limit specified in clause 9.9.3.5, starting from the beginning of time interval T2.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.6.13.2	PRS-RSRP reporting delay test case for dual positioning frequency layer

##### A.6.6.13.2.1	Test purpose and Environment

The purpose of the test is to verify that the PRS-RSRP measurement meets the delay requirements specified in clause 9.9.3.5 in an environment with AWGN propagation conditions.

The supported test configurations are specified in Table A.6.6.13.2.1-1.

Table A.6.6.13.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell on NR RF channel #1 in FR1. Cell 2 is a neighbour cell on a different NR RF channel #2 in FR1. The test consists of two consecutive time intervals, with duration of T1 and T2. Both cells transmit PRS during T2.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The general test parameters are listed in Table A.6.6.13.2.1-2, and cell specific test parameters are listed in Table A.6.6.13.2.1-3.

Table A.6.6.13.2.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 2: Cell 2 | Cell 1 and Cell 2 are on differnet positioning frequency layers |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |
|  |  | 2 | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| Measurement gap |  | 1, 2, 3 | GP#24 or GP#0 Note 1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | NA | OFF |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | 10 |  |
| NOTE 1: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

Table A.6.6.13.2.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 2 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 3 | PRS.2.4 FR1 |  | PRS.2.4 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -101 | -Infinity | -108 |
|  |  | 2 | -Infinity | -101 | -Infinity | -108 |
|  |  | 3 | -Infinity | -98 | -Infinity | -105 |
| Io | dBm/9.36 MHz | 1 | N/A | -68.28 | N/A | -69.63 |
|  | dBm/9.36 MHz | 2 |  | -68.28 |  | -69.63 |
|  | dBm/38.16 MHz | 3 |  | -62.19 |  | -62.19 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.13.2.2	Test Requirements

The UE shall perform and report the PRS-RSRP measurements for Cell 1 and Cell 2, within the time limit specified in clause 9.9.3.5, starting from the beginning of time interval T2.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.6.13.3	PRS-RSRP reporting delay test case for reduced number of samples

##### A.6.6.13.3.1	Test purpose and Environment

The purpose of the test is to verify that the PRS-RSRP measurement meets the delay requirements for reduced number of samples specified in clause 9.9.3.5 in an environment with AWGN propagation conditions.

The supported test configurations are specified in Table A.6.6.13.3.1-1.

Table A.6.6.13.3.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR1. The test consists of two consecutive time intervals, with duration of T1 and T2. Both cells transmit PRS during T2.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The general test parameters are listed in Table A.6.6.13.3.1-2, and cell specific test parameters are listed in Table A.6.6.13.3.1-3.

Table A.6.6.13.3.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |
|  |  | 2 | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| Measurement gap |  | 1, 2, 3 | GP#24 or GP#0 Note 1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | NA | OFF |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | 5 |  |
| NOTE 1: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

Table A.6.6.13.3.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.1 FR1 |  | PRS.1.1 FR1 |  |
|  |  | 2 | PRS.1.1 FR1 |  | PRS.1.1 FR1 |  |
|  |  | 3 | PRS.2.1 FR1 |  | PRS.2.1 FR1 |  |
| PRS BW |  | 1 | 48 PRBs |  | 48 PRBs |  |
|  |  | 2 | 48 PRBs |  | 48 PRBs |  |
|  |  | 3 | 48 PRBs |  | 48 PRBs |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -1.76 | -Infinity | -6.01 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | 0 | -Infinity | -3 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -98 | -Infinity | -101 |
|  |  | 2 | -Infinity | -98 | -Infinity | -101 |
|  |  | 3 | -Infinity | -95 | -Infinity | -98 |
| Io | dBm/9.36 MHz | 1 | N/A | -66.07 | N/A | -66.07 |
|  | dBm/9.36 MHz | 2 |  | -66.07 |  | -66.07 |
|  | dBm/38.16 MHz | 3 |  | -59.97 |  | -59.97 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |

##### A.6.6.13.3.2	Test Requirements

The UE shall perform and report the PRS-RSRP measurements for Cell 1 and Cell 2, within the time limit for reduced number of samples specified in clause 9.9.3.5, starting from the beginning of time interval T2.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.6.13.4	PRS-RSRP reporting delay test case for single positioning frequency layer outside MG

##### A.6.6.13.4.1	Test purpose and Environment

The purpose of the test is to verify that the PRS-RSRP measurement outside MG meets the delay requirements specified in clause 9.9.3.6 in an environment with AWGN propagation conditions. There are two sub-tests in the test, sub-test 1 is to verify the delay requirements with Nsample=1, and sub-test 2 is to verify the delay requirements with Nsample=4.

The supported test configurations are specified in Table A.6.6.13.4.1-1.

Table A.6.6.13.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR1. The test consists of two consecutive time intervals, with duration of T1 and T2. Both cells transmit PRS during T2.The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n. In sub-test 1, requestedDL-PRS-ProcessingSamples shall be included in the location information request and set to ‘m1’.

During T1, a PPW shall be configured for the PCell and be activated via DL MAC CE. The last PDSCH containing the MAC CE shall be transmitted before slot #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first PPW instance containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The general test parameters are listed in Table A.6.6.13.4.1-2, and cell specific test parameters during T2 are listed in Table A.6.6.13.4.1-3.

Table A.6.6.13.4.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |
|  |  | 2 | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| PPW configuration |  | 1 | TBD |  |
|  |  | 2 | TBD |  |
|  |  | 3 | TBD |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | NA | OFF |
| Time offset between serving and neighbour cells | s | 1 | 3 |  |
|  |  | 2 | 3 |  |
|  |  | 3 | 3 or 2.34 Note 1 |  |
| Expected RSTD | s | 1, 2, 3 | 0 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | Same as time offset between serving and neighbour cells |  |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | [5] |  |
| NOTE 1: If UE indicates support of CP length for the receive time difference threshold, the time offset is set to 2.34us, otherwise 3us. |  |  |  |  |

Table A.6.6.13.4.1-3: Cell specific test parameters during T2

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Sub-test 1 | Sub-test 2 | Sub-test 1 | Sub-test 2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.3 FR1 |  | PRS.1.3 FR1 |  |
|  |  | 2 | PRS.1.3 FR1 |  | PRS.1.3 FR1 |  |
|  |  | 3 | PRS.2.3 FR1 |  | PRS.2.3 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -3 | -3 | -6 | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -3 | -3 | -6 | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -101 | -101 | -104 | -108 |
|  |  | 2 | -101 | -101 | -104 | -108 |
|  |  | 3 | -98 | -98 | -101 | -105 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -101 | -101 | -104 | -108 |
|  |  | 2 | -101 | -101 | -104 | -108 |
|  |  | 3 | -98 | -98 | -101 | -105 |
| Io | dBm/9.36 MHz | 1 | -68.28 | -68.28 | -69.07 | -69.63 |
|  | dBm/9.36 MHz | 2 | -68.28 | -68.28 | -69.07 | -69.63 |
|  | dBm/38.16 MHz | 3 | -62.19 | -62.19 | -62.98 | -63.54 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP/PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.13.4.2	Test Requirements

The UE shall perform and report the PRS-RSRP measurements for Cell 1 and Cell 2, within the time limit specified in clause 9.9.3.6, starting from the beginning of time interval T2, with Nsample=1 for sub-test 1 and Nsample=4 for sub-test 2.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.6.6.14	UE Rx-Tx time difference measurements

#### A.6.6.14.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA

##### A.6.6.14.1.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx measurement meets the requirements specified in clause 9.9.4.5 in AWGN propagation condition in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations in listed in Table A.6.6.14.1.1-1.

Table A.6.6.14.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). Both cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. Cell 1 and Cell 2 mute PRS transmission during T1 and transmit PRS during T2.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the multi-RTT assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources.

The UE is configured with measurement gap pattern ID #0 or ID #24 before T2.

The UE is configured to transmit positioning SRS during T2.

The general test parameters and cell specific test parameters are as given in Table A.6.6.14.1.1-2 and Table A.6.6.14.1.1-3 respectively.

Table A.6.6.14.1.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| RF Channel Number |  | 1, 2, 3 | 1 | For both Cell 1 and Cell 2 |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |
|  |  | 2 | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| Measurement gap |  | 1, 2, 3 | GP#24 or GP#0 Note 1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | OFF |  |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 10 |  |
| Note 1: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

Table A.6.6.14.1.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 2 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 3 | PRS.2.2 FR1 |  | PRS.2.2 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| SRS configuration |  | 1 | POS-SRS.1 |  | N/A |  |
|  |  | 2 | POS-SRS.1 |  | N/A |  |
|  |  | 3 | POS-SRS.2 |  | N/A |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2.41 | -Infinity | -12.12 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -100 | -Infinity | -108 |
|  |  | 2 | -Infinity | -100 | -Infinity | -108 |
|  |  | 3 | -Infinity | -97 | -Infinity | -105 |
| Io | dBm/9.36 MHz | 1 | N/A | -67.67 | N/A | -67.67 |
|  | dBm/9.36 MHz | 2 |  | -67.67 |  | -67.67 |
|  | dBm/38.16 MHz | 3 |  | -61.57 |  | -61.57 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.14.1.1-4: Void

##### A.6.6.14.1.2	Test requirements

The UE Rx-Tx time difference measurement time fulfils the requirements specified in clause 9.9.4.5.

The UE shall perform and report the UE Rx-Tx time difference measurements for Cell 1 and Cell 2 within the specified UE Rx-Tx time difference measurement time starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause 10.1.25.3.1.

#### A.6.6.14.2	UE Rx-Tx time difference measurement for dual positioning frequency layers in FR1 SA

##### A.6.6.14.2.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx measurement meets the requirements specified in clause 9.9.4.5 in AWGN propagation condition in FR1 in standalone scenario when dual positioning frequency layers are configured.

The supported test configurations in listed in Table A.6.6.9.2.1-1.

Table A.6.6.14.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). Cell 1 and Cell2 are on different RF channels in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. Cell 1 and Cell 2 mute PRS transmission during T1 and transmit PRS during T2.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the multi-RTT assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources.

The UE is configured with measurement gap pattern ID #0 or ID #24 before T2.

The UE is configured to transmit positioning SRS during T2.

The general test parameters and cell specific test parameters are as given in Table A.6.6.14.2.1-2 and Table A.6.6.14.2.1-3 respectively.

Table A.6.6.14.2.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| RF Channel Number |  | 1, 2, 3 | 1 | For Cell 1 |
| RF Channel Number |  | 1, 2, 3 | 2 | For Cell 2 |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |
|  |  | 2 | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| Measurement gap |  | 1, 2, 3 | GP#24 or GP#0 Note 1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | OFF |  |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 10 |  |
| Note 1: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

Table A.6.6.14.2.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 2 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 3 | PRS.2.2 FR1 |  | PRS.2.2 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| SRS configuration |  | 1 | POS-SRS.1 |  | N/A |  |
|  |  | 2 | POS-SRS.1 |  | N/A |  |
|  |  | 3 | POS-SRS.2 |  | N/A |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | -13 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | -13 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -101 | -Infinity | -111 |
|  |  | 2 | -Infinity | -101 | -Infinity | -111 |
|  |  | 3 | -Infinity | -98 | -Infinity | -108 |
| Io | dBm/9.36 MHz | 1 | N/A | -68.28 | N/A | -69.84 |
|  | dBm/9.36 MHz | 2 |  | -68.28 |  | -69.84 |
|  | dBm/38.16 MHz | 3 |  | -62.19 |  | -63.74 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.14.2.1-4: Void

##### A.6.6.14.2.2	Test requirements

The UE Rx-Tx time difference measurement time fulfils the requirements specified in clause 9.9.4.5.

The UE shall perform and report the UE Rx-Tx time difference measurements for Cell 1 and Cell 2 within the specified UE Rx-Tx time difference measurement time starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause 10.1.25.3.1.

#### A.6.6.14.3	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with reduced sample number

##### A.6.6.14.3.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx measurement meets the requirements specified in clause 9.9.4.5 with Nsample = 1 in AWGN propagation condition in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations in listed in Table A.6.6.14.3.1-1.

Table A.6.6.14.3.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). All cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. Cell 1 and Cell 2 mute PRS transmission during T1 and transmit PRS during T2.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. requestedDL-PRS-ProcessingSamples shall be included in the location information request and set to ‘m1’. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the multi-RTT assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources.

The UE is configured with measurement gap pattern ID #0 or ID #24 before T2.

The UE is configured to transmit SRS during T2.

The general test parameters and cell specific test parameters are as given in Table A.6.6.14.3.1-2 and Table A.6.6.14.3.1-3 respectively.

Table A.6.6.14.3.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| RF Channel Number |  | 1, 2, 3 | 1 | For both Cell 1 and Cell 2 |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |
|  |  | 2 | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| Measurement gap |  | 1, 2, 3 | GP#24 or GP#0 Note 1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | OFF |  |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 10 |  |
| Note 1: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

Table A.6.6.14.3.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.1 FR1 |  | PRS.1.1 FR1 |  |
|  |  | 2 | PRS.1.1 FR1 |  | PRS.1.1 FR1 |  |
|  |  | 3 | PRS.2.1 FR1 |  | PRS.2.1 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| SRS configuration |  | 1 | POS-SRS.1 |  | N/A |  |
|  |  | 2 | POS-SRS.1 |  | N/A |  |
|  |  | 3 | POS-SRS.2 |  | N/A |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2 | -Infinity | -6 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2 | -Infinity | -6 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -100 | -Infinity | -104 |
|  |  | 2 | -Infinity | -100 | -Infinity | -104 |
|  |  | 3 | -Infinity | -97 | -Infinity | -101 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -100 | -100 | -104 | -104 |
|  |  | 2 | -100 | -100 | -104 | -104 |
|  |  | 3 | -97 | -97 | -101 | -101 |
| Io | dBm/9.36 MHz | 1 | N/A | -67.92 | N/A | -69.63 |
|  | dBm/9.36 MHz | 2 |  | -67.92 |  | -69.63 |
|  | dBm/38.16 MHz | 3 |  | -61.83 |  | -63.54 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.14.3.2	Test requirements

The UE Rx-Tx time difference measurement time fulfils the requirements specified in clause 9.9.4.5 with Nsample=1.

The UE shall perform and report the UE Rx-Tx time difference measurements for Cell 1 and Cell 2 within the specified UE Rx-Tx time difference measurement time starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause 10.1.25.3.1.

#### A.6.6.14.4	UE Rx-Tx time difference measurement without gaps in FR1 SA

##### A.6.6.14.4.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx measurement meets the requirements specified in clause 9.9.4.6 in AWGN propagation condition in FR1 in standalone scenario. There are two sub-tests in the test, sub-test 1 is to verify the delay requirements with Nsample=1, and sub-test 2 is to verify the delay requirements with Nsample=4.

The supported test configurations in listed in Table A.6.6.14.4.1-1.

Table A.6.6.14.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). All cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. Cell 1 and Cell 2 mute PRS transmission during T1 and transmit PRS during T2.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the multi-RTT assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of PRS processing window containing the PRS resources.

The UE is configured with PRS processing window before T2.

The UE is configured to transmit SRS during T2.

The general test parameters and cell specific test parameters are as given in Table A.6.6.14.4.1-2 and Table A.6.6.14.4.1-3 respectively.

Table A.6.6.14.4.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| RF Channel Number |  | 1, 2, 3 | 1 | For both Cell 1 and Cell 2 |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |
|  |  | 2 | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| PRS processing window |  | TBD | TBD |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | OFF |  |
| Time offset between serving and neighbour cells | s | 1 | 3 Note1 |  |
|  |  | 2 | 3 Note1 |  |
|  |  | 3 | 3 or 2.34 Note1 |  |
| Expected RSTD | s | 1,2,3 | 0 |  |
| Expected RSTD uncertainty | s | 1,2,3 | Same as time offset between serving and neighbour cells |  |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 10 |  |
| NOTE 1: The value is up to the UE capability. The possible UE capability value: (1/4 symbol, 1/2 symbol, CP length, half of slot). When the UE reported value is > 3us, the time offset between serving and neighbour cells is set to 3us; when the UE reported value is < 3us, the time offset between serving and neighbour cells is set to the UE reported value. |  |  |  |  |

Table A.6.6.14.4.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Sub-test 1 | Sub-test 2 | Sub-tets 1 | Sub-test 2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.1 FR1 |  | PRS.1.1 FR1 |  |
|  |  | 2 | PRS.1.1 FR1 |  | PRS.1.1 FR1 |  |
|  |  | 3 | PRS.2.1 FR1 |  | PRS.2.1 FR1 |  |
| PRS BW |  |  | 48 PRBs | 24 PRBs | 48 PRBs | 24 PRBs |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| SRS configuration |  | 1 | POS-SRS.1 |  | N/A |  |
|  |  | 2 | POS-SRS.1 |  | N/A |  |
|  |  | 3 | POS-SRS.2 |  | N/A |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -3 | -2.41 | -6 | -12.12 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -1.44 | -2 | -3.65 | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -99.44 | -100 | -101.65 | -108 |
|  |  | 2 | -99.44 | -100 | -101.65 | -108 |
|  |  | 3 | -96.44 | -97 | -98.65 | -105 |
| Io | dBm/9.36 MHz | 1 | -66.73 | -67.67 | -66.73 | -67.67 |
|  | dBm/9.36 MHz | 2 | -66.73 | -67.67 | -66.73 | -67.67 |
|  | dBm/38.16 MHz | 3 | -60.62 | -61.57 | -60.62 | -61.57 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.14.4.2	Test requirements

The UE Rx-Tx time difference measurement time fulfils the requirements specified in clause 9.9.4.6.

The UE shall perform and report the UE Rx-Tx time difference measurements for Cell 1 and Cell 2 within the specified UE Rx-Tx time difference measurement time starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause 10.1.25.3.1.

#### A.6.6.14.5	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with multiple RxTx TEGs

##### A.6.6.14.4.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx measurement meets the requirements specified in clause 9.9.4.5 in AWGN propagation condition in FR1 in standalone scenario when single positioning frequency layer is configured, and when UE is requested to measure a PRS resource with multiple RxTx TEGs.

The supported test configurations in listed in Table A.6.6.14.4.1-1.

Table A.6.6.14.4.11: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). All cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. Cell 1 and Cell 2 mute PRS transmission during T1 and transmit PRS during T2.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the multi-RTT assistance data and location information request. In nr-Multi-RTT-RequestLocationInformation, measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 shall be set to ‘n2’.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources.

The UE is configured with measurement gap pattern ID #0 or ID #24 before T2.

The UE is configured to transmit SRS during T2.

The general test parameters and cell specific test parameters are as given in Table A.6.6.14.4.1-2 and Table A.6.6.14.4.1-3 respectively.

Table A.6.6.14.4.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| RF Channel Number |  | 1, 2, 3 | 1 | For both Cell 1 and Cell 2 |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |
|  |  | 2 | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| Measurement gap |  | 1, 2, 3 | GP#24 or GP#0 Note 1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | OFF |  |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 10 |  |
| Note 1: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

Table A.6.6.14.4.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 2 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 3 | PRS.2.2 FR1 |  | PRS.2.2 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| SRS configuration |  | 1 | POS-SRS.1 |  | N/A |  |
|  |  | 2 | POS-SRS.1 |  | N/A |  |
|  |  | 3 | POS-SRS.2 |  | N/A |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2.41 | -Infinity | -12.12 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -100 | -Infinity | -108 |
|  |  | 2 | -Infinity | -100 | -Infinity | -108 |
|  |  | 3 | -Infinity | -97 | -Infinity | -105 |
| Io | dBm/9.36 MHz | 1 | N/A | -67.67 | N/A | -67.67 |
|  | dBm/9.36 MHz | 2 |  | -67.67 |  | -67.67 |
|  | dBm/38.16 MHz | 3 |  | -61.57 |  | -61.57 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.14.4.2	Test requirements

The UE Rx-Tx time difference measurement time fulfils the requirements specified in clause 9.9.4.5, with kmultiTEG=2 if UE does not support or indicate value ‘n1’ for measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul, and  kmultiTEG=1 otherwise.

The UE shall perform and report the UE Rx-Tx time difference measurements for Cell 1 and Cell 2 within the specified UE Rx-Tx time difference measurement time starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause 10.1.25.3.1.

### A.6.6.15	Idle Mode measurements of inter-RAT CA candidate cells for early reporting

#### A.6.6.15.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly retains the detected cell status for the idle mode CA measurement when UE transitions from RRC Connected mode to Idle mode, when the UE has entered Idle mode. Additionally, test that the UE performs the required measurements on the serving cell and the configured inter-RAT carrier for idle mode measurement reporting. This test will partly verify the Idle mode CA measurements in clause 4.4. In the test, connected mode DRX configuration is not configured in either PCell or PSCell.

Additionally, the purpose of this test is to verify that the SS-RSRP, SS-RSRQ, RSRP and RSRQ measurement accuracy is within the specified limits. This test will verify the accuracy requirements in Sections 10.1.2B and 10.1.7B for intra-frequency measurements and section 10.2.2 and 10.2.3 for the inter-RAT measurements for the supported test configurations in tables A.6.6.15.1-4 and A.6.6.15.1-5.

The supported test configurations are given in Table A.6.6.15.1-1. The test parameters are given in Tables A.6.6.15.1-2, A.6.6.15.1-3, A.6.6.15.1-4 and A.6.6.15.1-5 below. In the test there are two cells, cell 1, which is the PCell in connected, and serving cell in idle mode, on radio channel 1 in FR1, and cell 2, which is the PSCell in connected, and measured LTE inter-RAT cell in idle mode, on radio channel 2 in LTE.

For the purpose of testing absolute accuracy in idle mode in this set of test cases the cells in idle mode are on different carrier frequencies (NR FR1 and LTE).  The absolute accuracy of RSRP and RSRQ inter-RAT measurements are tested by using the parameters in Table A.6.6.15.1-4 and Table A.6.6.15.1-5. In all test cases, Cell 1 is the serving and Cell 2 the target cell.

The test consists of 5 successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. During T1 cell 2, the PSCell, shall be configured.

Time duration T2 starts when UE has transmitted random access preamble on the PSCell. After T2, the UE is configured with idle mode CA measurements with the PSCell carrier as the target carrier. The connection is released 500 ms after T2 when the UE has sent random access preamble on the PSCell.

T3 starts when the connection is released. During the time periods T3 and T4 the UE is in Idle mode with the serving cell on the FR1 carrier. The UE is configured to perform inter-RAT idle mode CA/DC measurements on Cell 2 carrier. After the connection release and during T3, 1000 ms after T3 is started, the signal level of the inter-RAT carrier configured for idle mode CA/DC measurements is changed at which time T4 starts. T5 starts [65]s after T4, when the UE is paged for connection setup and UE is requested by the network to report idle mode CA/DC measurements.

Table A.6.6.15.1-1: Supported test configurations for Idle Mode measurements of inter-frequency CA candidate cells for early reporting

| Config | Description |
| --- | --- |
| 1 | FR1 FDD SSB SCS 15kHz BW 10MHz – LTE FDD 10MHz |
| 2 | FR1 FDD SSB SCS 15kHz BW 10MHz – LTE TDD 10MHz |
| 3 | FR1 TDD SSB SCS 30kHz BW 40MHz – LTE FDD 10MHz |
| 4 | FR1 TDD SSB SCS 30kHz BW 40MHz – LTE TDD 10MHz |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.6.15.1-2: General test parameters for Idle Mode measurements of inter-frequency CA candidate cells for early reporting

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1 in FR1 |
| PSCell |  | Cell 2 | PSCell on RF channel number 2 in LTE |
| DRX |  | OFF | For both PCell and PSCell once configured |
| PRACH configuration in Cell 2 |  | PRACH_2CE | PRACH configuration as specified in Clause A.3.16 in TS 36.133 |
| T331 | s | 300 |  |
| CSI reporting periodicity and offset configuration for Cell 2 | ms | 2 |  |
| T1 | s | 0.5 | During this time the PCell is known and PSCell is configured. |
| T2 | s | 0.5 | PSCell access. |
| T3 + T4 | s | 66 | During this time the UE is configured to perform inter-frequency measurements in idle mode on the PSCell carrier. |
| T5 | s | 0.5 | UE is paged and connection is setup. Network requests measurement report from the UE. |

Table A.6.6.15.1-3: Cell specific test parameters for NR cell for Idle Mode measurements of inter-frequency CA candidate cells for early reporting

| Parameter | Unit | Config | Test 1 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | T1 | T2 | T3 |  |  |  | T4 |  |  |  | T5 |
| AoA setup |  | 1,2,3,4 | N/A |  |  |  |  |  |  |  |  |  |  |
| Assumption for UE beams Note 5 R: Rough |  | 1,2,3,4 | N/A | N/A | R |  |  |  | R |  |  |  | N/A |
| Frequency Range |  | 1,2,3,4 | FR1 |  |  |  |  |  |  |  |  |  |  |
| Duplex mode |  | 1, 2 | FDD |  |  |  |  |  |  |  |  |  |  |
|  |  | 3, 4 | TDD |  |  |  |  |  |  |  |  |  |  |
| TDD Confguration 1: TDDConf.1.1 2: TDDConf.2.1 |  | 1,2 | - |  |  |  |  |  |  |  |  |  |  |
|  |  | 3,4 | 1 | 1 | 2 |  |  |  | 2 |  |  |  | 1 |
| BWchannel 1: 10: NRB,c = 52 2: 40: NRB,c = 106 | MHz | 1, 2 | 1 | 1 | - |  |  |  | - |  |  |  | 1 |
|  |  | 3, 4 | 2 | 2 | - |  |  |  | - |  |  |  | 2 |
| Initial Downlink BWP configuration |  | 1,2,3,4 | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |  |
| Initial Uplink BWP configuration |  | 1,2,3,4 | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |
| Dedicated Downlink BWP configuration 1: DLBWP.1.1 |  | 1,2,3,4 | 1 | 1 | - |  |  | - |  |  |  |  | 1 |
| Dedicated Uplink BWP configuration 1: ULBWP.1.1 |  | 1,2,3,4 | 1 | 1 | - |  |  | - |  |  |  |  | 1 |
| PDSCH Reference Measurement Channel 1: SR.1.1 FDD 2: SR.2.1 TDD | FDD | 1,2 | 1 | 1 | 1 |  |  | 1 |  |  |  |  | 1 |
|  | TDD | 3,4 | 2 | 2 | 2 |  |  | 2 |  |  |  |  | 2 |
| TRS configuration |  | 1,2,3,4 | - |  |  |  |  |  |  |  |  |  |  |
| TCI state |  | 1,2,3,4 | - |  |  |  |  |  |  |  |  |  |  |
| RMSI CORESET parameters | FDD | 1,2 | CR.1.1 FDD |  |  |  |  |  |  |  |  |  |  |
|  | TDD | 3,4 | CR.2.1 TDD |  |  |  |  |  |  |  |  |  |  |
| Dedicated CORESET parameters | FDD | 1,2 | CCR.1.1 FDD |  |  |  |  |  |  |  |  |  |  |
|  | TDD | 3,4 | CCR.2.1 TDD |  |  |  |  |  |  |  |  |  |  |
| OCNG PatternsNote1 |  | 1,2,3,4 | OP.1 defined in A.3.2.1 |  |  |  |  |  |  |  |  |  |  |
| SSB configuration 1: SSB.1 FR1 2: SSB.2 FR1 |  | 1,2 | 1 |  |  |  |  |  |  |  |  |  |  |
|  |  | 3,4 | 2 |  |  |  |  |  |  |  |  |  |  |
| SMTC configuration |  | 1,2,3,4 | SMTC.2 |  |  |  |  |  |  |  |  |  |  |
| Correlation Matrix and Antenna config |  | 1,2,3,4 | 1x2 Low |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS | dB | 1,2,3,4 | 0 | 0 | - |  |  | - |  |  |  |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  | 0 | 0 | - |  |  | - |  |  |  |  | 0 |
| EPRE ratio of PBCH to PBCH DMRS |  |  | 0 | 0 | - |  |  | - |  |  |  |  | 0 |
| EPRE ratio of PDCCH DMRS to SSS |  |  | 0 | 0 | - |  |  | - |  |  |  |  | 0 |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | 0 | 0 | - |  |  | - |  |  |  |  | 0 |
| EPRE ratio of PDSCH DMRS to SSS |  |  | 0 | 0 | - |  |  | - |  |  |  |  | 0 |
| EPRE ratio of PDSCH to PDSCH |  |  | 0 | 0 | - |  |  | - |  |  |  |  | 0 |
| EPRE ratio of OCNG DMRS to SSS |  |  | 0 | 0 | - |  |  | - |  |  |  |  | 0 |
| Noc Note2 | dBm/ 15kHz | 1,2 | -98 | -98 | -98 |  |  | -98 |  |  |  |  | -98 |
|  |  | 3,4 |  |  |  |  |  |  |  |  |  |  |  |
| Noc Note2 | dBm/SCS | 1,2 | -98 | -98 | -98 |  | -98 |  |  |  |  | -98 |  |
|  |  | 3,4 | -95 | -95 | -95 |  | -95 |  |  |  |  | -95 |  |
| Ês/Iot | dB | 1,2,3,4 | 5 | 5 | 5 |  | 5 |  |  |  |  | 5 |  |
| Ês/Noc | dB | 1,2,3,4 | 5 | 5 | 5 |  | 5 |  |  |  |  | 5 |  |
| SS-RSRPNote3,4 | dBm/SCS | 1,2 | -93 | -93 | -93 |  | -93 |  |  |  |  | -93 |  |
|  |  | 3,4 | -90 | -90 | -90 |  | -90 |  |  |  |  | -90 |  |
| IoNote3,4 | dBm/ 9.36 MHz | 1,2 | -63.85 | -63.85 | -63.85] | -63.85 |  |  |  |  | -63.85 |  |  |
|  | dBm/ 38.16 MHz | 3,4 | -57.76 | -57.76 | -57.76 | -57.76 |  |  |  |  | -57.76 |  |  |
| Qrxlevmin | dBm/SCS | 1, 2 | - | - | -140 |  |  |  |  | - |  |  |  |
|  |  | 3, 4 | - | - | -137 |  |  |  |  | - |  |  |  |
| Pcompensation | dB | 1,2,3,4 | - | - | 0 | 0 |  |  |  | - |  |  |  |
| Qhysts | dB | 1,2,3,4 | - | - | 0 | 0 |  |  |  | - |  |  |  |
| Qoffsets, n | dB | 1,2,3,4 | - | - | 0 | 0 |  |  |  | - |  |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1,2,3,4 | SS-RSRP |  |  |  |  |  |  |  |  |  |  |
| Treselection | s | 1,2,3,4 | - |  | 0 |  |  |  |  | - |  |  |  |
| SnonintrasearchP | dB | 1,2,3,4 | - |  | Not Sent |  |  |  |  | - |  |  |  |
| SnonintrasearchQ | dB | 1,2,3,4 | - |  | Not Sent |  |  |  |  | - |  |  |  |
| Threshx, high | dB | 1,2,3,4 | - |  | [48] |  |  |  |  | - |  |  |  |
| Threshserving, low | dB | 1,2,3,4 | - |  | [44] |  |  |  |  | - |  |  |  |
| Threshx, low | dB | 1,2,3,4 | - |  | [50] |  |  |  |  | - |  |  |  |
| Propagation Condition | dB | 1,2,3,4 | - |  | AWGN |  |  |  |  | - |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SSB_RP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.6.6.15.1-4: Cell specific test parameters for LTE cell for Idle Mode measurements of inter-frequency CA candidate cells for early reporting

| Parameter | Unit | Config | Test 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 2 |  |  |  |  |  |
|  |  |  | T1 | T2 | T3 | T4 |  | T5 |
| Frequency Range |  | 1,2,3,4 | LTE |  |  |  |  |  |
| Duplex mode |  | 1, 3 | FDD |  |  |  |  |  |
|  |  | 2, 4 | TDD |  |  |  |  |  |
| BWchannel | MHz | 1,2,3,4 | 10 | 10 | - | - |  | 10 |
| Measurement bandwidth |  | 1,2,3,4 | - | - | 22-27 | 22-27 |  | - |
| PDSCH Reference Measurement Channel 1: R.1 FDD 2: R.1 TDD | FDD | 1,3 | 1 | 1 | - | - |  | 1 |
|  | TDD | 2,4 | 2 | 2 | - | - |  | 2 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 and A.3.1.2.2 in 36.133 1: R.6 FDD 2: R.6 TDD |  | 1,3 | 1 | 1 | - | - |  | 1 |
|  |  | 2,4 | 2 | 2 | - | - |  | 2 |
| OCNG Patterns defined in A.3.2.1.1 (OP.2 FDD) and A.3.2.1.2 (OP.2 TDD) in 36.133 1: OP.2 FDD 2: OP.2 TDD |  | 1,3 | 1 |  |  |  |  |  |
|  |  | 2,4 | 2 |  |  |  |  |  |
| Correlation Matrix and Antenna config |  | 1,2,3,4 | 1x2 Low |  |  |  |  |  |
| PBCH_RA | dB | 1,2,3,4 | N/A | N/A | 0 | 0 |  | N/A |
| PBCH_RB |  |  | N/A | N/A | 0 | 0 |  | N/A |
| PSS_RA |  |  | N/A | N/A | 0 | 0 |  | N/A |
| SSS_RA |  |  | N/A | N/A | 0 | 0 |  | N/A |
| PCFICH_RB |  |  | N/A | N/A | 0 | 0 |  | N/A |
| PHICH_RA |  |  | N/A | N/A | 0 | 0 |  | N/A |
| PHICH_RB |  |  | N/A | N/A | 0 | 0 |  | N/A |
| PDCCH_RA |  |  | N/A | N/A | 0 | 0 |  | N/A |
| PDCCH_RB |  |  | N/A | N/A | 0 | 0 |  | N/A |
| PDSCH_RA |  |  | N/A | N/A | 0 | 0 |  | N/A |
| PDSCH_RB |  |  | N/A | N/A | 0 | 0 |  | N/A |
| OCNG_RANote 1 |  |  | N/A | N/A | 0 | 0 |  | N/A |
| OCNG_RBNote 1 |  |  | N/A | N/A | 0 | 0 |  | N/A |
| Noc Note2 | dBm/ 15kHz | 1,2 | -98 | -98 | -98 | -98 |  | -98 |
|  |  | 3,4 |  |  |  |  |  |  |
| Ês/Iot | dB | 1,2,3,4 | 5 | 5 | -3 | 8 |  | 5 |
| Ês/Noc | dB | 1,2,3,4 | 5 | 5 | -3 | 8 |  | 5 |
| SS-RSRPNote3,4 | dBm/SCS | 1,2,3,4 | -93 | -93 | -101 | -90 |  | -93 |
| IoNote3,4 | dBm/ 9.36 MHz | 1,2, 3, 4 | -61.01 | -61.01 | -65.44 | -58.57 |  | -61.01 |
| Qrxlevmin | dBm/SCS | 1,2, 3, 4 | - | - | -140 |  |  | - |
| Pcompensation | dB | 1,2,3,4 | - | - | 0 | 0 |  | - |
| Qhysts | dB | 1,2,3,4 | - | - | 0 | 0 |  | - |
| Qoffsets, n | dB | 1,2,3,4 | - | - | 0 | 0 |  | - |
| Cell_selection_and_ reselection_quality_measurement |  | 1,2,3,4 | RSRP and RSRQ |  |  |  |  |  |
| Treselection | s | 1,2,3,4 | - |  | 0 |  | - |  |
| SnonintrasearchP | dB | 1,2,3,4 | - |  | Not Sent |  | - |  |
| SnonintrasearchQ | dB | 1,2,3,4 | - |  | Not Sent |  | - |  |
| Threshx, high | dB | 1,2,3,4 | - |  | [48] |  | - |  |
| Threshserving, low | dB | 1,2,3,4 | - |  | [44] |  | - |  |
| Threshx, low | dB | 1,2,3,4 | - |  | [50] |  | - |  |
| Propagation Condition | dB | 1,2,3,4 | - |  | AWGN |  | - |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SSB_RP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |

Table A.6.6.15.1-5: General idle mode test parameters for Idle Mode measurements of inter-frequency CA candidate cells for early reporting

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Serving cell |  | 1, 2, 3, 4 | Cell1 | The UE camps on cell 1 which is the former PCell. |
| Neighbour cell |  | 1, 2, 3, 4 | Cell2 | The UE shall perform inter-frequency measurements on cell 2 which is the former PSCell. |
| RF Channel Number |  | 1, 2, 3, 4 | 1, 2 |  |
| Time offset between cells |  | 1, 2, 3, 4 | 3 s | Synchronous cells |
| Access Barring Information | - | 1, 2, 3,4 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  | 1, 2 | SSB.1 FR1 | Serving cell |
|  |  | 3, 4 | SSB.2 FR1 | Serving cell |
| SMTC configuration Serving cell |  | 1, 2, 3, 4 | SMTC.2 |  |
| DRX cycle length | s | 1, 2, 3, 4 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  | 1, 2, 3, 4 | 190 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  | 1, 2, 3, 4 | Not configured |  |
| T3 | s | 1, 2, 3, 4 | 0.5 | T3 needs to be defined so that cell measurement time is taken into account. |
| T4 | s | 1, 2, 3, 4 | 65 | T4 needs to be defined so that cell measurement time is taken into account. |

#### A.6.6.15.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During time durations T1 the UE shall start transmitting preamble on PSCell. During T2 the UE perform intra-frequency measurements on the PCell and the PSCell.

During the time-period T3 the connection is released, and UE enters idle mode. During the time period T3 and T4 the UE is camped in Idle mode and at T4 the signal level of cell 2 is changed. The UE shall not perform reselection. The UE shall perform Idle Mode CA measurement according to Section 4.4.

At the start of T5 the UE is paged for connection setup. During the connection setup the UE is requested to transmit early measurement report. The UE shall send early measurement report to the PCell including idle mode CA/DC measurement from cell 2.

After receiving the requested early measurement report, the test equipment verifies that the accuracy of measurement reported for serving Cell 1 and Cell 2 meets the requirements in Sections 10.1.2B and 10.1.7B and Sections 10.2.4 and 10.2.5, respectively and test ends.

### A.6.6.16	PRS-RSRPP measurements

#### A.6.6.16.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state

##### A.6.6.16.1.1	Test purpose and Environment

The purpose of the test is to verify that the PRS-RSRPP measurement meets the delay requirements specified in clause 9.9.6.5 in an environment with AWGN propagation conditions.

The supported test configurations are specified in Table A.6.6.16.1.1-1.

Table A.6.6.16.1.11: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR1. The test consists of two consecutive time intervals, with duration of T1 and T2. Both cells transmit PRS during T2.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The general test parameters are listed in Table A.6.6.16.1.1-2, and cell specific test parameters are listed in Table A.6.6.16.1.1-3.

Table A.6.6.16.1.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |
|  |  | 2 | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| Measurement gap |  | 1, 2, 3 | GP#24 or GP#0 Note 1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | NA | OFF |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | [5] |  |
| NOTE 1: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

Table A.6.6.16.1.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 2 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 3 | PRS.2.4 FR1 |  | PRS.2.4 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -101 | -Infinity | -108 |
|  |  | 2 | -Infinity | -101 | -Infinity | -108 |
|  |  | 3 | -Infinity | -98 | -Infinity | -105 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -88 | -88 | -88 | -88 |
|  |  | 2 | -88 | -88 | -88 | -88 |
|  |  | 3 | -85 | -85 | -85 | -85 |
| Io | dBm/9.36 MHz | 1 | N/A | -62.25 | N/A | -62.25 |
|  | dBm/9.36 MHz | 2 |  | -62.25 |  | -62.25 |
|  | dBm/38.16 MHz | 3 |  | -56.16 |  | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP/PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.16.1.2	Test Requirements

The UE shall perform and report the PRS-RSRPP measurements for Cell 1 and Cell 2, within the time limit specified in clause 9.9.6.5, starting from the beginning of time interval T2.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.6.16.2	PRS-RSRPP reporting delay test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state

##### A.6.6.16.2.1	Test purpose and Environment

The purpose of the test is to verify that the PRS-RSRPP measurement meets the reduced sample measurement delay requirements specified in clause 9.9.6.5 in an environment with AWGN propagation conditions for reduced number of samples. In this test UE that supports supportedDL-PRS-ProcessingSamples is configured by LMF to perform PRS measurement with reduced number of samples.

The supported test configurations are specified in Table A.6.6.16.2.1-1.

Table A.6.6.16.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR1. The test consists of two consecutive time intervals, with duration of T1 and T2. Both cells transmit PRS during T2.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The general test parameters are listed in Table A.6.6.16.2.1-2, and cell specific test parameters are listed in Table A.6.6.16.2.1-3.

Table A.6.6.16.2.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |
|  |  | 2 | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| Measurement gap |  | 1, 2, 3 | GP#24 or GP#0 Note 1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | NA | OFF |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | [5] |  |
| NOTE 1: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

Table A.6.6.16.2.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 2 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 3 | PRS.2.4 FR1 |  | PRS.2.4 FR1 |  |
| PRS BW |  | 1,2,3 | 48 PRBs |  | 48 PRBs |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | 5 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | 5 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -101 | -Infinity | -93 |
|  |  | 2 | -Infinity | -101 | -Infinity | -93 |
|  |  | 3 | -Infinity | -98 | -Infinity | -90 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -88 | -88 | -88 | -88 |
|  |  | 2 | -88 | -88 | -88 | -88 |
|  |  | 3 | -85 | -85 | -85 | -85 |
| Io | dBm/9.36 MHz | 1 | N/A | -69.5 | N/A | -67.9 |
|  | dBm/9.36 MHz | 2 |  | -69.5 |  | -67.9 |
|  | dBm/38.16 MHz | 3 |  | -66.4 |  | -64.8 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP/PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.16.2.2	Test Requirements

The UE shall perform and report the PRS-RSRPP measurements for Cell 1 and Cell 2, within the time limit specified in clause 9.9.6.5, starting from the beginning of time interval T2.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.6.16.3	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state without measurement gap

##### A.6.6.16.3.1	Test purpose and Environment

The purpose of the test is to verify that the PRS-RSRPP measurement meets the delay requirements specified in clause 9.9.6.6 in an environment with AWGN propagation conditions. Reporting delay test for gapless PRS measurement is conducted assuming that the PRS has higher priority, i.e., state 1, than all other DL signals/channels and is transmitted within active DL BWP of UE. Two sub-tests are defined, sub-test 1 is for Nsample = 4 and sub-test 2 is for Nsample = 1. For sub-test 2 LMF indicates UE to perform PRS measurement with Nsample = 1. The cell specific parameters for sub-test 1 and sub-test 2 are defined in Table A.6.6.16.3.1-3.

The supported test configurations are specified in Table A.6.6.16.3.1-1.

Table A.6.6.16.3.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR1. The test consists of two consecutive time intervals, with duration of T1 and T2. Both cells transmit PRS during T2.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first PRS processing window instance containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The UE is configured with PPW before start of T2.

The general test parameters are listed in Table A.6.6.16.3.1-2, and cell specific test parameters are listed in Table A.6.6.16.3.1-3.

Table A.6.6.16.3.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |
|  |  | 2 | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| PPW configuration |  | 1,2,3 | Table A.3.X-1: Reference PPW configuration | As defined in A.3.X |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | NA | OFF |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | - set to the UE reported capability for receive time difference threshold if the UE reported value is < 3µs - 3µs otherwise | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 0 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | Same as time offset |  |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | ms | 1, 2, 3 | 10 | In this test UE is configured to measure single PFL within the configured PPW. |

Table A.6.6.16.3.1-3: Cell specific test parameters T2

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Sub-test 1 | Sub-test 2 | Sub-test 1 | Sub-test 2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 2 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 3 | PRS.2.4 FR1 |  | PRS.2.4 FR1 |  |
| PRS configuration |  |  | 48 PRBs for sub-test 2 |  | 48 PRBs for sub-test 2 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -3 | -3 | -10 | 5 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -3 | -3 | -10 | 5 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -101 | -101 | -108 | -93 |
|  |  | 2 | -101 | -101 | -108 | -93 |
|  |  | 3 | -98 | -98 | -105 | -90 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -88 | -88 | -88 | -88 |
|  |  | 2 | -88 | -88 | -88 | -88 |
|  |  | 3 | -85 | -85 | -85 | -85 |
| Io | dBm/9.36 MHz | 1 | -62.25 | -62.25 | -62.25 | -67.9 |
|  | dBm/9.36 MHz | 2 | -62.25 | -62.25 | -62.25 | -67.9 |
|  | dBm/38.16 MHz | 3 | -56.16 | -56.16 | -56.16 | -61.8 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP/PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.16.3.2	Test Requirements

The UE shall perform and report the PRS-RSRPP measurements for Cell 1 and Cell 2, within the time limit specified in clause 9.9.6.6, starting from the beginning of time interval T2.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.6.6.17	SA event triggered reporting tests with Pre-MG

#### A.6.6.17.1	SA event triggered reporting tests with autonomous activation/deactivation Pre-MG

##### A.6.6.17.1.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clause 9.2.6.2 and 9.2.6.3. And this test will also jointly verify Pre-configured measurement gap activation/deactivation delay in clause 8.19.2.

##### A.6.6.17.1.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell are given in Table A.6.6.17.1.2-1 and A.6.6.17.1.2-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively.

During the duration of T1,

-	UE is configured with 2 different UE-specific bandwidth parts for Cell 1 (PCell), BWP-1 and BWP-2, before starting the test.

-	BWP-1 includes bandwidth of the initial DL BWP and SSB with the Pre-MG status set to ‘deactivated’.

-	BWP-2 does not include bandwidth of the initial DL BWP and SSB with the Pre-MG status set to ‘activated’.

UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 in PCell.

-	At the start of time duration T2, the serving gNB can trigger Pre-MG activation starts when a DCI format 1_1 command for PCell DL BWP switch, sent from the test equipment to the UE, is received at the UE side in PCell’s slot # denoted i. The UE shall switch its bandwidth part from BWP-1 to BWP-2. And UE is expected to complete the Pre-MG activation within T2.

-	At the start of time duration T3, the UE may not have any timing information of neighbor cell to be measured (e.g. cell 2). And UE was expected to complete the measurements of SSBs with the activated Pre-MG within T3.

There are two BWPs configured in Cell 1, BWP-1 which contains the cell defining SSB, and BWP-2 which does not contain any SSB of Cell 1.

Table A.6.6.17.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.17.1.2-2: General test parameters for SA intra-frequency event triggered reporting with per-UE gaps for PCell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| Measurement gap type |  | 1, 2, 3 | Per-UE gaps |  |
| Measurement gap epetition periodicity | ms | 1, 2, 3 | 40 |  |
| Measurement gap length | ms | 1, 2, 3 | 6 |  |
| Measurement gap offset | ms | 1, 2, 3 | 39 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CSI-RS parameters |  | 1 | CSI-RS.1.2 FDD resource #0 |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX | ms | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 0.1 |  |
| T2 | s | 1, 2, 3 | 0.2 |  |
| T3 | s | 1,2, 3 | 5 |  |
| bwp-InactivityTimer | ms |  | 500 |  |

Table A.6.6.17.1.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting with per-UE gaps for PCell in FR1

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1 | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| Intial BWP configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| BWP-1 Configuration |  | 1,2,3 | DLBWP.1.6 ULBWP.1.6 |  |  | N/A |  |  |
| BWP-2 Configuration |  | 1,2,3 | DLBWP.1.5 ULBWP.1.5 |  |  | N/A |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |  | N/A |  |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
|  |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.2 FDD |  |  | CCR.1.1 FDD |  |  |
|  |  | 2 | CCR.1.2 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  |  | OP.1 |  |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  |  | N/A |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  | N/A |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  | N/A |  |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.2 |  |  | DLBWP.1.1 |  |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.2 |  |  | ULBWP.1.1 |  |  |
| RLM-RS |  | 1, 2, 3 | CSI-RS |  |  | SSB |  |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
|  | dB | 1 | 4 | -1.46 |  | -Infinity |  | -1.46 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
|  | dB | 1 | 4 | 4 |  | -Infinity |  | 4 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 |  | -Infinity |  | -94 |
|  |  | 2 | -94 | -94 |  | -Infinity |  | -94 |
|  |  | 3 | -91 | -91 |  | -Infinity |  | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 |  | -64.60 |  | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 |  | -64.60 |  | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 |  | -58.50 |  | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

##### A.6.6.17.1.3	Test Requirements

During T1 and T2 the UE shall report corresponding valid ACK/NACK for those PDSCHs scheduled in the slots overlapped with the Pre-MG occasions, starting from the 1st complete Pre-MG occasion after the beginning of PCell’s DL slot (i+TBWPswitchDelay) + 5ms as defined in clause 8.19.2.

During T3, the UE shall NOT be able to receive PDSCH and report corresponding valid ACK/NACK for those PDSCHs scheduled in the slots overlapped with the Pre-MG occasions.

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800ms from the beginning of time period T3.

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.17.2	SA event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation

##### A.6.6.17.2.1	Test purpose and Environment

The purpose of this test is to verify that the UE correctly activates and deactivates the pre-MG and makes correct measurement and reporting of an event with activated and deactivated pre-MG. This test will partly verify the pre-MG activation and deactivation delay requirements in clause 8.19.2 and the intra-frequency cell search requirements in clause 9.2.6.2 and 9.3.4.

##### A.6.6.17.2.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The supported test configurations are listed in Table A.6.6.17.2.2-1, general test parameters are listed in Table A.6.6.17.2.2-2, and cell specific test parameters are listed in Table A.6.6.17.2.2-3.

In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. A pre-MG is configured before the test.

The UE is configured with 2 dedicated BWPs, BWP-1 and BWP-2. BWP-1 includes bandwidth of the SSB, and preConfGapStatus for BWP-1 is set to ‘0’; BWP-2 does not include bandwidth of the SSB, and preConfGapStatus for BWP-2 is set to ‘1’.

The test consists of 3 successive time periods, with time duration of T1, T2, and T3 respectively. Before the test starts, UE is switched to BWP-1.

The time period T2 starts when a DCI format 1_1 command for PCell DL BWP switch, sent from the test equipment to the UE, is received at the UE side in PCell’s slot # denoted i. The UE shall switch its bandwidth part from BWP-1 to BWP-2 and complete pre-MG activation during T2.

Time period T3 starts at the beginning of the first complete MG occasion after slot # i+k, where k is number of slots corresponding to the length of DCI based BWP switch delay plus 5ms. During T3, UE shall perform intra-frequency measurement with pre-MG activated.

During time duration T1 and T2, the UE shall not have any timing information of Cell 2.

Table A.6.6.17.2.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.17.2.2-2: General test parameters for SA intra-frequency event triggered reporting with with pre-configured measurement gaps and network-controlled activation/deactivation

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| Measurement gap type |  | 1, 2, 3 | Per-UE gaps |  |
| Measurement gap repitition periodicity | ms | 1, 2, 3 | 40 |  |
| Measurement gap length | ms | 1, 2, 3 | 6 |  |
| Measurement gap offset | ms | 1, 2, 3 | 39 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CSI-RS parameters |  | 1 | CSI-RS.1.2 FDD resource #0 |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX | ms | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 0.1 |  |
| T2 | s | 1, 2, 3 | 0.1 |  |
| T3 | s | 1, 2, 3 | 5 |  |

Table A.6.6.17.2.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting with with pre-configured measurement gaps and network-controlled activation/deactivation

| Parameter | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |  | T1 | T2 | T3 |
| TDD configuration |  | 1 | N/A |  |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |  |  | N/A |  |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |  |  | N/A |  |  |
|  |  | 2 | CR.1.1 TDD |  |  |  | N/A |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  | N/A |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.2 FDD |  |  |  | N/A |  |  |
|  |  | 2 | CCR.1.2 TDD |  |  |  | N/A |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  | N/A |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  |  |  | OP.1 |  |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  |  |  | N/A |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  | N/A |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  | N/A |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  |  |  | N/A |  |  |
| Active DL BWP configuration for BWP-1 |  | 1, 2, 3 | DLBWP.1.3 |  |  |  | N/A |  |  |
| Active UL BWP configuration for BWP-1 |  | 1, 2, 3 | ULBWP.1.3 |  |  |  | N/A |  |  |
| Active DL BWP configuration for BWP-2 |  | 1, 2, 3 | DLBWP.1.2 |  |  |  | N/A |  |  |
| Active UL BWP configuration for BWP-2 |  | 1, 2, 3 | ULBWP.1.2 |  |  |  | N/A |  |  |
| RLM-RS |  | 1, 2, 3 | CSI-RS |  |  |  | N/A |  |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |
|  | dB | 1,2,3 | -1.46 | 4 |  | -1.46 | -1.46 | -Infinity | -1.46 |
|  | dB | 1,2,3 | 4 | 4 |  | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1,2 | -94 | -94 |  | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 |  | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1,2 | -64.60 | -64.60 |  | -62.25 | -62.25 | -64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -58.50 |  | -56.16 | -56.16 | -58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2 and T4. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |

##### A.6.6.17.2.3	Test Requirements

During T1, UE shall report corresponding HARQ-ACK for those PDSCHs scheduled in the slots overlapped with the pre-MG occasions.

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T3. During T3, UE is not required to report corresponding HARQ-ACK for those PDSCHs scheduled in the slots overlapped with the pre-MG occasions.

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.17.3	Void

##### A.6.6.17.3.1	Void

##### A.6.6.17.3.2	Void

##### A.6.6.17.3.3	Void

### A.6.6.18	SA event triggered reporting tests with concurrent gaps

#### A.6.6.18.1	SA event triggered reporting tests for FR1 concurrent gaps with non-overalpping scenario for SSB-based measurements in both inter-frequency layers

##### A.6.6.18.1.1	Test Purpose and Environment

The purpose of this test is to verify that the concurrent gaps capable UE makes correct reporting of events. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are three cells: NR cell 1 as PCell in FR1 on NR RF channel 1, NR cell 2 as neighbour cell in FR1 on NR RF channel 2, and NR cell 3 as neighbour cell in FR1 on NR RF channel 3.  The test parameters are given in Tables A.6.6.18.1.1-1, A.6.6.18.1.1-2 and A.6.6.18.1.1-3.

Two measurement gap patterns (MeasGapId #0 and MeasGapId #1) are configured with the gap pattern ID #0 and #1 as defined in Table A.6.6.18.1.1-2. MeasGapId #1 is configured with a higher priority than MeasGapId #0. MeasGapId #0 and MeasGapId #1 are associated with the MOs for RF channel numbers #1 and #2, respectively.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used for both frequency layers. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2 and NR cell 3.

Table A.6.6.18.1.1-1: SA event triggered reporting tests for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: Target NR cells have the same SCS, BW and duplex mode as NR serving cells |  |

Table A.6.6.18.1.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 concurrent gaps with fully non-overalpping scenario for SSB-based measurements in both inter-frequency layers

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2, 3 |  | Three FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cells 2 and 3 |  | NR cell 2 is on NR RF channel number 2. NR cell 3 is on NR RF channel number 3. |
| Gap Pattern Id |  | Config 1,2,3 | 0 for MeasGapId #0 1 for MeasGapId #1 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 39 for MeasGapId #0 19 for MeasGapId #1 |  |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cell 1 |  | Config 1, 2, 3 | 3s |  | Synchronous. |
| Time offset between serving and neighbour cell 2 |  | Config 1, 2, 3 | [5ms] |  | [Asynchronous. The timing of Cell 3 is 5ms later than the timing of Cell 1.] |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 1.5 | 1.5 |  |

Table A.6.6.18.1.13: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 concurrent gaps with fully non-overalpping scenario for SSB-based measurements in both inter-frequency layers

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  | 3 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | CR.2.1 TDD |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |  |  |
| SSB parameters |  |  | Config 1,2,3 | SSB.1 FR1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1,2,3 | SMTC.2 |  | SMTC.2 |  | SMTC.2 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 | -Infinity | -88 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 | -70.05 | -62.26 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |

##### A.6.6.18.1.2	Test Requirements

The UE shall send one Event A3 triggered measurement report for each neighboring cell, with a measurement reporting delay less than 920 ms for cell 2 and 1280ms for cell 3 from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.18.2	SA event triggered reporting tests for FR1 concurrent gap with partially partial overalpping scenario for SSB-based measurements in both inter-frequency layers

##### A.6.6.18.2.1	Test Purpose and Environment

The purpose of this test is to verify that the concurrent gap capable UE makes correct reporting of events. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are three cells: NR cell 1 as PCell in FR1 on NR RF channel 1, NR cell 2 as neighbour cell in FR1 on NR RF channel 2, and NR cell 3 as neighbour cell in FR1 on NR RF channel 3.  The test parameters are given in Tables A.6.6.18.2.1-1, A.6.6.18.2.1-2 and A.6.6.18.2.1-3. The TE schedules continuous DL data on PCell throughout the test.

Two measurement gap patterns (MeasGapId #1 and MeasGapId #2) are configured with the gap pattern ID #0 and #1 as defined in Table A.6.6.18.2.1-2. MeasGapId #2 is configured with a higher priority than MeasGapId #1. MeasGapId #1 and MeasGapId #2 are associated with the MOs for RF channel numbers #2 and #3, respectively.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used for both frequency layers. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2 and NR cell 3.

Table A.6.6.18.2.1-1: SA event triggered reporting tests for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: Target NR cells have the same SCS, BW and duplex mode as NR serving cells |  |

Table A.6.6.18.2.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 concurrent gap with partially partial overalpping scenario for SSB-based measurements in both inter-frequency layers

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2, 3 |  | Three FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cells 2 and 3 |  | NR cell 2 is on NR RF channel number 2. NR cell 3 is on NR RF channel number 3. |
| Gap Pattern Id |  | Config 1,2,3 | 0 for MeasGapId #1 1 for MeasGapId #2 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset | ms | Config 1,2,3 | 39 for MeasGapId #1 4 for MeasGapId #2 |  |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cell 1 |  | Config 1, 2, 3 | 3s |  | Synchronous. |
| Time offset between serving and neighbour cell 2 |  | Config 1, 2, 3 | 5ms |  | Asynchronous. The timing of Cell 3 is 5ms later than the timing of Cell 1. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 1.5 | 1.5 |  |

Table A.6.6.18.2.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 concurrent gap with partially-partial overalpping scenario for SSB-based measurements in both inter-frequency layers

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  | 3 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | CR.2.1 TDD |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |  |  |
| SSB parameters |  |  | Config 1,2,3 | SSB.1 FR1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1,2,3 | SMTC.2 |  | SMTC.2 |  | SMTC.Y |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 | -Infinity | -88 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 | -70.05 | -62.26 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |

##### A.6.6.18.2.2	Test Requirements

The UE shall send one Event A3 triggered measurement report for each neighboring cell, with a measurement reporting delay less than 1280 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.18.3	SA NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1

##### A.6.6.18.3.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE makes correct event-triggered reporting of concurrent inter-RAT E-UTRAN and NR FR1 measurements when operating in standalone (SA) operation with PCell in FR1. This test shall partly verify the cell search and measurement requirements in Clauses 9.4.2, 9.4.3, 9.3.4 and 9.3.5.

In each test there are three cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the NR PCell, Cell 2 is an Inter-frequency NR FR1 neighbour cell on NR RF channel 2 and Cell 3 is an inter-RAT E-UTRAN neighbour cell.

In the measurement control information from the PCell it is indictated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) is to be used for the E-UTRAN cell (cell 3). In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A4 is used for the NR FR1 cell (cell 2).

Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2 and Cell 3.

In the test two concurrent per-UE measurement gap pattern configurations # 0 as defined in Table A.6.6.18.3.1-2 are provided for a UE. Two measurement gap patterns (MeasGapId #0 and MeasGapId #1) are configured with the gap pattern ID #0 as defined in Table A.6.6.18.3.1-2. MeasGapId #1 is configured with a higher priority than MeasGapId #0. MeasGapId #0 and MeasGapId #1 are associated with the MOs for RF channel numbers #1 and #2, respectively.

Supported test configurations are shown in table A.6.6.18.3.1-1. General test parameters are provided in Table A.6.6.18.3.1-2 below. Test parameters for Cell 1, Cell 2 and Cell 3, valid for both time duration T1 and T2, are provided in Tables A.6.6.18.3.1-3 and A.6.6.18.3.1-4, respectively.

The test parameters and configurations are given in Tables A.6.6.18.3.1-1, A.6.6.18.3.1-2, and A.6.6.18.3.1-3.

Table A.6.6.18.3.1-1: Supported test configurations in SA concurrent inter-RAT E-UTRAN and NR FR2 inter-frequency event triggered reporting in non-DRX with PCell in FR1

| Configuration | Description | Description of target cell |
| --- | --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE: FDD. NR: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE: FDD. NR: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE: FDD. NR: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE: TDD. NR: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE: TDD. NR: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE: TDD. NR: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

Table A.6.6.18.3.1-2: General test parameters for SA concurrent inter-RAT E-UTRAN and NR FR2 inter-frequency event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | 1 - 6 | 1, 2 | 2 NR carrier frequency is used in the test. 1 FR1 and FR2 carrier frequency. |
| LTE RF Channel Number |  |  |  | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | 1 - 6 | As specified in Tables A.6.6.3.1.1-2 and A.6.6.3.1.1-3. |  |
| Active cell |  | 1 - 6 | NR Cell 1 (PCell in FR1) | Cell 1 is on RF channel number 1 |
| Neighbour cell 1 |  | 1 - 6 | Cell 2 (NR FR2) | Cell 2 is on RF channel number 2 |
| Neighbour cell 2 |  | 1 - 6 | Cell 3 (LTE) | Cell 3 is on RF channel number 3 |
| Gap Pattern Id #1 |  | 1 - 6 | 0 | As specified in Clause Table 9.1.2-1. Per-UE gap pattern. |
| Gap Pattern Id #2 |  | 1 - 6 | 0 | Gap Pattern Id #2 is offsetted 20ms compared to Gap Pattern Id #1 |
| Measurement gap offset |  | 1 – 6 | 19 for Gap pattern Id #1 |  |
| NR measurement quantity |  |  | SS-RSRP | Measurement quantity for Cell 1 |
| NR measurement quantity |  |  | SS-RSRP | Measurement quantity for Cell 2 |
| Inter-RAT E-UTRAN measurement quantity |  |  | RSRP | Measurement quantity for Cell 3 |
| CSI-RS for tracking parameters on NR RF Channel 1 |  | 1, 4 | TRS.1.1 FDD |  |
|  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  | 3, 6 | TRS.1.2 TDD |  |
| SMTC-SSB parameters on NR RF Channel 1 |  | 1, 4 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | 2, 5 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | 3, 6 | SSB.2 FR1 | As specified in clause A.3.10.1 |
| SMTC-SSB parameters on NR RF Channel 2 |  | 1 - 6 | SSB.3 FR2 | As specified in clause A.3.10.2 |
| b2-Threshold1 | dBm |  | Note 1 | SS-RSRP threshold for SS-RSRP measurement on Cell 3 for event B2 |
| b2-Threshold2EUTRA | dBm |  | -95 | E-UTRAN RSRP threshold for SS-RSRP measurement on Cell 3 for event B2 |
| Hysteresis | dB | 1 – 6 | 0 |  |
| TimeToTrigger | s | 1 – 6 | 0 |  |
| Filter coefficient |  | 1 – 6 | 0 | L3 filtering is not used |
| offsetMO | dB | 1 - 6 | 6 | NR Cell 2 |
| a4-Threshold | dBm | 1 – 6 | -105 | NR Cell 2 |
| DRX |  |  | OFF | OFF |
| Time offset between serving and neighbour cells |  | 1, 4 | 3ms | Asynchronous cells NR cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2, 3, 5, 6 | 3s | Synchronous NR cells. |
| T1 | s | 1 – 6 | 5 | for LTE Cell 3 and NR FR2 Cell 2 |
| T2 | s | 1 - 6 | 5 | for LTE Cell 3 |
|  |  |  | 5.2 for PC1; 3.5 for other PC | for NR FR2 Cell 2 |

Table A.6.6.18.3.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Config 1,2,3 | N/A |  | Setup 1 as specified in clause A.3.15 |  |
| Beam AssumptionNote 7 |  |  | Config 1,2,3 | N/A |  | Rough |  |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  | TDD |  |
|  |  |  | Config 2,3 | TDD |  | TDD |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  | TDDConf.3.1 |  |
|  |  |  | Config 2 | TDDConf.1.1 |  | TDDConf.3.1 |  |
|  |  |  | Config 3 | TDDConf.2.1 |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | Config 1 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 2 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  |  | Config 1 | 52 |  | 66 |  |
|  |  |  | Config 2 | 52 |  | 66 |  |
|  |  |  | Config 3 | 106 |  | 66 |  |
| BWP BW |  | MHz | Config 1 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 2 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1,2,3 | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference |  |  | Config 1 | SR.1.1 FDD |  | - |  |
| measurement channel |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET Reference |  |  | Config 1 | CR.1.1 FDD |  | - |  |
| Channel |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  |  | Config 1 | CCR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SMTC configuration defined |  |  | Config 1 | SMTC.2 |  | SMTC.2 |  |
| in A.3.11.1 and A.3.11.2 |  |  | Config 2,3 | SMTC.1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  | 120 |  |
|  |  |  | Config 3 | 30 |  | 120 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Ês |  | dBm/SCS | Config 1,2,3 |  |  | -Infinity | -87 |
| SSB_RP Note 3 |  | dBm/SCS | Config 1,2 |  |  | -Infinity | -87 |
|  |  | Note5 | Config 3 |  |  | -Infinity | -87 |
| BB Note 8 |  | dB | Config 1,2,3 | NA Link only, see clause |  | -Infinity | 14.69 |
|  |  | dBm/95.04 MHz Note5 | Config 1,2,3 | A.3.7A |  | -Infinity | -58.01 |
| Propagation Condition |  |  | Config 1,2,3 |  |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: SS B_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 8: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |

Table A.6.6.18.3.1-4: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Configuration | Cell 3 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -76.22+10log (NRB,c /50) | -59.13+10log (NRB,c /50) |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.6.6.18.3.2	Test Requirements

In this test with per-UE gap, the UE shall send one Event A4 triggered measurement report for Cell 2, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

5120 for UE supporting power class 1, or

3200 for UE supporting other power class.

The UE shall send one Event B2 triggered measurement report for Cell 3 to the PCell, with a measurement reporting delay less than 3.84s from the start of period T2.

The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.18.4	SA event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used

##### A.6.6.18.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA NR measurements with concurrent gaps requirements in clause 9.2.6(when one of concurrent gaps in same frequency layer of serving cells), 9.3.6(when one of concurrent gaps in the different frequency layer of serving cells) and 9.9.2(when one of concurrent gaps used for PRS measurement).

In this test, there are three cells: NR cell 1 as PCell in FR1 on NR RF channel 1 , NR cell 2 as neighbour cell in FR1 on NR RF channel 2 and NR cell 3 as neighbor cell in FR1 on NR RF channel 1, and NR cell 3 as neighbour cell in FR1 on NR RF channel 1.  The test parameters are given in Tables A.6.6.18.4.1-1, A.6.6.18.4.1-2 and A.6.6.18.4.1-3.

Two measurement gap patterns (MeasGapId #0 and MeasGapId #1) are configured with the gap pattern ID #0 and #1 as defined in Table A.6.6.18.4.1-2. MeasGapId #1 is configured with a higher priority than MeasGapId #0.  MeasGapId #0 and MeasGapId #1 are associated with the MOs for RF channel numbers #1 and #2, respectively.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2 and NR cell 3. Cell 1 and cell 3 transmit PRS during T2.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance of MeasGapId #1 containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

Table A.6.6.18.4.1-1: SA event triggered reporting tests without SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.18.4.1-2: General test parameters for SA inter-frequency event triggered reporting for concurrent gaps with partially partial overalpping scenario for SSB-based measurements and PRS measurement

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | 1,2,3 | 1: Cell 1 and Cell 3 2: Cell 2 | Two TDD carrier frequencies are used for the NR cells. |
| Active cell |  | 1,2,3 | NR cell 1 (Pcell) | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1,2,3 | NR cell 2, NR cell 3 | Cell 2 is an inter-frequency cell neighbor cell Cell 3 is a neighbour cell in the positioning assistance data. |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |
|  |  | 2 | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| Gap Pattern Id |  | Config 1,2,3 | 0 for MeasGapId #0 24 for MeasGapId #1 |  |
| Measurement gap offset | ms | Config 1,2,3 | 7 for MeasGapId #0 11 for MeasGapId #1 |  |
| DRX |  | 1, 2, 3 | NA | OFF |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | 5 |  |
| NOTE 1: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

Table A.6.6.18.4.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 concurrent gap with partially-partial overalpping scenario for SSB-based measurements and PRS measurement

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |  | T1 | T2 |
| TDD configuration |  |  | 1 | N/A |  | N/A |  |  | N/A |  |
|  |  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |  | TDDConf.1.1 |  |
|  |  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |  | TDDConf.2.1 |  |
| BWchannel |  | MHz | Config 1,2,3 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |
| BWP BW |  | MHz | Config 1,2,3 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1,2,3 | DLBWP.0.1 |  | N/A |  |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |  | OP.1 |  |
| PDSCH RMC configuration |  |  | 1 | SR.1.1 FDD |  | N/A |  |  | N/A |  |
|  |  |  | 2 | SR.1.1 TDD |  |  |  |  |  |  |
|  |  |  | 3 | SR.2.1 TDD |  |  |  |  |  |  |
| RMSI CORESET RMC configuration |  |  | 1 | CR.1.1 FDD |  | N/A |  |  | N/A |  |
|  |  |  | 2 | CR.1.1 TDD |  | N/A |  |  | N/A |  |
|  |  |  | 3 | CR.2.1 TDD |  | N/A |  |  | N/A |  |
| Dedicated CORESET RMC configuration |  |  | 1 | CCR.1.1 FDD |  | N/A |  |  | N/A |  |
|  |  |  | 2 | CCR.1.1 TDD |  | N/A |  |  | N/A |  |
|  |  |  | 3 | CCR.2.1 TDD |  | N/A |  |  | N/A |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR2 |  | SSB.1 FR2 |  |  | SSB.1 FR2 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.4 |  | SMTC.4 |  |  | SMTC.4 |  |
| TRS Configuration |  |  | 1 | TRS.1.1 FDD |  | N/A |  |  | N/A |  |
|  |  |  | 2 | TRS.1.1 TDD |  | N/A |  |  | N/A |  |
|  |  |  | 3 | TRS.1.2 TDD |  | N/A |  |  | N/A |  |
| PRS configuration |  |  | 1 | PRS.1.4 FR1 |  | N/A |  |  | PRS.1.4 FR1 |  |
|  |  |  | 2 | PRS.1.4 FR1 |  | N/A |  |  | PRS.1.4 FR1 |  |
|  |  |  | 3 | PRS.2.4 FR1 |  | N/A |  |  | PRS.2.4 FR1 |  |
| PRS muting configuation |  |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |  | ‘01’ |  |
| Note2 |  | dBm/15kHz Note5 | 1 | -98 |  | -98 |  |  | -98 |  |
|  |  |  | 2 | -98 |  | -98 |  |  | -98 |  |
|  |  |  | 3 | -98 |  | -98 |  |  | -98 |  |
| Note2 |  | dBm/SCS Note4 | 1 | -98 |  | -98 |  |  | -98 |  |
|  |  |  | 2 | -98 |  | -98 |  |  | -98 |  |
|  |  |  | 3 | -95 |  | -95 |  |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS Note5 | 1 | -94 | -94 | -Infinity |  | -94 | N/A | N/A |
|  |  |  | 2 | -94 | -94 | -Infinity |  | --94 | N/A | N/A |
|  |  |  | 3 | -91 |  |  |  |  |  |  |
| PRS-RSRP Note 3 |  | dBm/SCS Note5 | 1 | -Infinity | --101 | N/A |  | N/A | -Infinity | -108 |
|  |  |  | 2 | -Infinity | --101 | N/A |  | N/A | -Infinity | -108 |
|  |  |  | 3 | -Infinity | --98 | N/A |  | N/A | -Infinity | -105 |
| PRS |  | dB | 1,2,3 | -Infinity | -3 | N/A |  | N/A | -Infinity | -10 |
| PRS |  | dB | 1,2,3 | -Infinity | -3 | N/A |  | N/A | -Infinity | -10 |
| IoNote3 |  | dBm/95.04 MHz Note5 | 1 | -62.25 |  | --64.60 |  | -62.25 | -62.25 |  |
|  |  |  | 2 | -62.25 |  | --64.60 |  | -62.25 | -62.25 |  |
|  |  |  | 3 | -56.16 |  | --58.50 |  | -56.16 | -56.16 |  |
| Propagation Condition |  |  | 1,2,3 | AWGN |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP/PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |  |  |

##### A.6.6.18.4.2	Test Requirements

The UE shall send one Event A3 triggered measurement report for cell 2, with a measurement reporting delay less than 1840ms from the beginning of time period T2.

The PRS RSRP measurement time fulfils the requirements specified in Clause 9.9.3.5. The UE shall perform and report the PRS RSRP measurements for Cell 3 with respect to the reference cell in the DL-AoD assistance data, Cell 1, within the time duration specified in section 9.9.3.5 starting from the beginning of time interval T2.

The rate of the correct events for the neighbour cell observed during repeated tests shall be at least 90%, where the reported PRS RSRP measurement for each correct event shall be within the PRS RSRP reporting range specified in Clause 10.1.24.3, i.e., between PRS RSRP_0 and PRS RSRP_126.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

IUE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.6.6.19	SA event triggered reporting tests with NCSG

#### A.6.6.19.1	SA event triggered reporting tests with NCSG under non-DRX in FR1

##### A.6.6.19.1.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clause 9.2.7.1 and 9.2.7.2, and also verify the scheduling availability during intra-frequency measurement with NCSG in clause 9.2.7.3.

The serving frequency should be selected for which UE reports ‘ncsg’.

##### A.6.6.19.1.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell are given in Table A.6.6.19.1.2-1, A.6.6.19.1.2-2 and A.6.6.19.1.2-3 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

During T2, the UE is continuously scheduled with data on the PCell.

The UE is capable of NCSG and report ‘ncsg’ through NeedForGapNCSG-InfoNR for PCell.

In test 1 NCSG pattern configuration # 0 as defined in Table A.6.6.19.1.2-2 is provided for UE that does not support per-FR gap and in test 2 NCSG pattern configuration #2 as defined in Table A.6.6.19.1.2-2 is provided for UE that supports per-FR gap. If a UE supports per-FR gap, it is only required to pass test 2. Otherwise it is only required to pass test 1.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

Table A.6.6.19.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.19.1.2-2: General test parameters for SA intra-frequency event triggered reporting with per-UE gaps for PCell in FR1

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1, 2, 3 | Cell 1 |  |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 |  | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |  |
| NCSG type |  | 1, 2, 3 | Per-UE | Per-FR | Per-FR NCSG pattern is configured if UE support per-FR NCSG; otherwise, per-UE NCSG patter is configured. |
| NCSG pattern |  | 1, 2, 3 | 0 | 2 |  |
| Visible interruption repetition periodicity | ms | 1, 2, 3 | 40 | 40 |  |
| Measurement length | ms | 1, 2, 3 | 5 | 2 |  |
| NCSG offset | ms | 1, 2, 3 | 39 | 39 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |  |
|  |  | 2 | SSB.1 FR1 |  |  |
|  |  | 3 | SSB.2 FR1 |  |  |
| SMTC configuration |  | 1 | SMTC.2 |  |  |
|  |  | 2 | SMTC.1 |  |  |
|  |  | 3 | SMTC.1 |  |  |
| CSI-RS parameters |  | 1 | CSI-RS.1.2 FDD resource #0 |  |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |  |
| CP length |  | 1, 2, 3 | Normal |  |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |  |
| Filter coefficient |  | 1, 2, 3 | 0 |  | L3 filtering is not used |
| DRX | ms | 1, 2, 3 |  |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s |  | Synchronous cells |
|  |  | 3 | 3 s |  | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |  |
| T2 | s | 1, 2, 3 | 1 |  |  |

Table A.6.6.19.1.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting with NCSG for PCell in FR1

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  | N/A |  |
|  |  | 3 | CR.2.1 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.2 FDD |  | N/A |  |
|  |  | 2 | CCR.1.2 TDD |  | N/A |  |
|  |  | 3 | CCR.2.1 TDD |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | CSI-RS |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | --58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.19.1.3	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

During T2, UE shall send HARQ ACK/NACK for the corresponding PDSCH scheduled in PCell in all the slots except for the case where PDSCH or PUCCH is overlapped with the VIL of NCSG pattern.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.19.2	SA event triggered reporting tests for FR1 with NCSG for inter-frequency measurement

##### A.6.6.19.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.10.

The serving frequency and the target frequency should be selected such that UE reports ‘ncsg’ for the target frequency given the serving frequency.

##### A.6.6.19.2.2	Test parameters

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2. NR RF channel 1 and NR RF channel 2 should be selected such that UE reports ‘ncsg’ for the target frequency on NR RF channel 2.

The test parameters are given in Tables A.6.6.19.2.2-1, A.6.6.19.2.2-2 and A.6.6.19.2.2-3.

In test 1 measurement NCSG pattern configuration # 0 as defined in Table A.6.6.19.2.2-2 is provided for UE that does not support per-FR NCSG, and in test 2 NCSG pattern configuration #2 as defined in Table A.6.6.19.2.2-2 is provided for UE that supports per-FR NCSG. If a UE supports per-FR NCSG, it is only required to pass test 2, otherwise it is only required to pass test 1.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

During T2, the UE is continuously scheduled with data on the PCell.

Table A.6.6.19.2.2-1: SA event triggered reporting tests for FR1 with NCSG for inter-frequency measurement

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.19.2.2-2: General test parameters for event triggered reporting for FR1 with NCSG for inter-frequency measurement

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 |  | NR cell 2 is on NR RF channel number 2. |
| NCSG Pattern Id |  | Config 1,2,3 | 0 | 2 | As specified in Table 9.1.9.3-1. |
| NCSG offset |  | Config 1,2,3 | 9 |  |  |
| SMTC-SSB parameters |  | Config 1 | SSB.1 FR1 |  | As specified in clause A.3.10.1 |
|  |  | Config 2 | SSB.1 FR1 |  | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.2 FR1 |  | As specified in clause A.3.10.1 |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 1 |  |  |

Table A.6.6.19.2.2-3: Cell specific test parameters for SA event triggered reporting for FR1 with NCSG for inter-frequency measurement

| Parameter |  | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  | -98 |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |  |
|  |  |  | Config 3 | -95 |  | -95 |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 |  | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 |  | -Infinity | -88 |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 |  | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 |  | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 |  | -70.05 | -62.26 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 |  | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |

##### A.6.6.19.2.3	Test Requirements

In test 1 and 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

During T2, UE shall send HARQ-ACK for the corresponding PDSCH scheduled in PCell in all the slots except for the case where PDSCH or PUCCH is overlapped with the VIL of NCSG pattern.

The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2, the UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.19.3	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG

##### A.6.6.19.3.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements based on NCSG when operating in standalone (SA) operation with PCell in FR1. This test shall partly verify the cell search and measurement requirements in Clauses 9.4.2 and 9.4.3.

The serving frequency and the target frequency should be selected such that UE reports ‘ncsg’ for the target frequency given the serving frequency.

##### A.6.6.19.3.2	Test parameters

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell. In the measurement control information from the PCell it is indictated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2.

Supported test configurations are shown in table A.6.6.19.3.2-1. General test parameters are provided in Table A.6.6.19.3.2-2 below. Test parameters for Cell 1 and Cell 2, valid for both time duration T1 and T2, are provided in Tables A.6.6.19.3.2-3 and A.6.6.19.3.2-4, respectively.

Table A.6.6.19.3.2-1: Supported test configurations in SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.6.19.3.2-2: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 1 | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in Tables A.6.6.3.1.1-2 and A.6.6.3.1.1-3. |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| NCSG Pattern Id |  | 0 | As specified in clause Table 9.1.9.3-1. |
| NCSG offset | ms | 39 |  |
| NCSG mgta | ms | 0 |  |
| NR measurement quantity |  | SS-RSRP | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | Note 1 | SS-RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| b2-Threshold2EUTRA | dBm | -95 | E-UTRAN RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| Note 1: Values are defined in Table A.6.6.3.1.1-3 |  |  |  |

Table A.6.6.19.3.2-3: PCell specific test parameters for SA inter-RAT E-UTRA event triggered reporting in non-DRX with PCell in FR1

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 2, 3 | FDD |  |
|  |  |  |  | 4, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |
|  |  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORSET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORSET reference channel |  |  |  | 1, 4 | CCR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CCR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  |  | 3, 6 | TRS.1.2 TDD |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | --96 |  |
|  |  |  |  | 3, 6 | --93 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -104 |  |
|  |  |  |  | 3, 6 | -101 |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 116 | 70 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 116 | 70 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | --88 | --104 |
|  |  |  |  | 3, 6 | --85 | --101 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | --88 | --104 |
|  |  |  |  | 3, 6 | --85 | --101 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | --59.94 | --73.04 |
|  |  |  | dBm/38.16 MHz | 3, 6 | --53.84 | --66.93 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | TDL-C 300ns 100Hz |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.19.3.2-4: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -76.22+10log (NRB,c /50) | -59.13+10log (NRB,c /50) |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | ETU70 |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.6.6.19.3.3	Test Requirements

The UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 3.84s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

During T2, UE shall send HARQ-ACK for the corresponding PDSCH scheduled in PCell in all the slots except for the case where PDSCH or PUCCH is overlapped with the VIL of NCSG pattern.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.19.4	Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX

##### A.6.6.19.4.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the cell search requirements on SCC with deactivated SCell in clauses 9.2.7.1 and 9.2.7.2.

##### A.6.6.19.4.2	Test parameters

Three cells are deployed in the test, which are FR1 PCell (Cell 1), FR1 SCell (Cell 2) and FR1 neighbour cell (Cell 3) on the same frequency as the SCell. The SCell is deactivated during the test. The test parameters for PCell, the SCell and the neighbour cell are given in Table A.6.6.19.4.2-1 and A.6.6.19.4.2-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A6 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 3. The PCell shall continuously scheduled with data in the DL starting from T1 until the UE has sent the measurement report during T2.

Table A.6.6.19.4.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.19.4.2-2: General test parameters for event triggered reporting on SCC with deactivated SCell with per-UE NCSG for FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Configured deactivated SCell |  | 1, 2, 3 | Cell 2 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 3 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 |  |
| RF Channel Number |  | 1, 2, 3 | 2: Cell 2 and Cell 3 |  |
| Measurement gap type |  | 1, 2, 3 | Per-UE NCSG |  |
| NCSG pattern |  | 1, 2, 3 | ID # 0 | Defined in Table 9.1.9.3-1 |
| MGTA | ms | 1, 2, 5 | 0 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A6-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 |  | OFF |
| SCell measurement cycle (measCycleSCell) | ms | 1, 2, 3 | 160 |  |
| Cell 2 timing offset to Cell 1 | s | 1, 2, 3 | 0 |  |
| Time alignment error between Cell 2 and Cell 1 | s | 1, 2, 3 | ≤ Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| Cell 3 timing offset to Cell 1 | s | 1, 2, 3 | 3 | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |

Table A.6.6.19.4.2-3: NR Cell specific test parameters for event triggered reporting on SCC with deactivated SCell with per-UE NCSG for FR1

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  | N/A |  | N/A |  |
|  |  | 3 | CR.2.1 TDD |  | N/A |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  | N/A |  | N/A |  |
|  |  | 3 | CCR.2.1 TDD |  | N/A |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | TRS.1.1 FDD |  |  |  |
|  |  | 2 | TRS.1.1 TDD |  | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | TRS.1.2 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  | N/A |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Note 2 | dBm/15 kHz | 1, 2, 3 | -98 |  |  |  |  |  |
|  | dB | 1, 2, 3 | 4 | 4 | 4 | -1.46 | -Infinity | -1.46 |
|  | dB | 1, 2, 3 | 4 | 4 | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -64.60 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -64.60 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -58.50 | -58.50 | -56.16 | -58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

##### A.6.6.19.4.3	Test Requirements

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 1600 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, at least 100% of all expected ACK/NACKs shall be transmitted by the UE.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

For a test to be considered successful requirements on both Event A6 detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.6.6.20	UE Rx-Tx time difference measurement for propagation delay compensation

#### A.6.6.20.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx measurement for RTT-based PDC meets the requirements specified in clause 9.12.4.1 for measurement delay and clause 10.1.X for measurement accuracy in AWGN propagation condition in FR1 in standalone scenario.

The supported test configurations in listed in Table A.6.6.20.1-1.

Table A.6.6.20.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

The test is considered with one cell (Cell 1) in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. If the test is based on PRS, the Cell 1 mutes PRS transmission during T1 and transmits PRS during T2.

If the test is based on PRS, the MeasObjectRxTxDiff-r17 with prs-Ref-r17 , measObject  with measObjectRxTxDiff-17, and NR-DL-PRS-PDC-Info as defined in TS 38.331 shall be provided to the UE during T1.

The last TTI containing the RRC configuration shall be provided to the UE T ms before the start of T2, where T = [TBD] ms.

The beginning of the time interval T2 shall be aligned with the beginning of the first PRS resources.

The UE is configured to transmit SRS during T2.

The general test parameters and cell specific test parameters are as given in Table A.6.6.20.1-2. The test parameters for PRS are given Table A.6.6.20.1-3.

Table A.6.6.20.1-2: General test parameters

| Parameter | Unit | Test configuration | Value |
| --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |
| RF Channel Number |  | 1, 2, 3 | 1 |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |
|  |  | 2 | 10: NRB,c = 52 |
|  |  | 3 | 40: NRB,c = 106 |
| SSB configuration |  | 1 | SSB.1 FR1 |
|  |  | 2 | SSB.1 FR1 |
|  |  | 3 | SSB.2 FR1 |
| SMTC configuration |  | 1 | SMTC.2 |
|  |  | 2 | SMTC.1 |
|  |  | 3 | SMTC.1 |
| CP length |  | 1, 2, 3 | Normal |
| DRX |  | 1, 2, 3 | OFF |
| T1 | s | 1, 2, 3 | 5 |
| T2 | s | 1, 2, 3 | 10 |

Table A.6.6.20.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1 | N/A |  |
|  |  | 2 | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |
|  |  | 2 | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  |
|  |  | 2 | TRS.1.1 TDD |  |
|  |  | 3 | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  |
| PRS configuration |  | 1 | PRS.1.2 FR1 |  |
|  |  | 2 | PRS.1.2 FR1 |  |
|  |  | 3 | PRS.2.2 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  |
| SRS configuration |  | 1 | [PDC-SRS.1] |  |
|  |  | 2 | [PDC-SRS.1] |  |
|  |  | 3 | [PDC-SRS.2] |  |
| Note 2 | dBm/SCS | 1 | -98 |  |
|  |  | 2 | -98 |  |
|  |  | 3 | -95 |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |
|  |  | 2 | -98 |  |
|  |  | 3 | -98 |  |
| PRS | dB | 1 | -Infinity | -2.41 |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRS | dB | 1 | -Infinity | -2 |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -100 |
|  |  | 2 | -Infinity | -100 |
|  |  | 3 | -Infinity | -97 |
| Io | dBm/9.36 MHz | 1 | N/A | -67.67 |
|  | dBm/9.36 MHz | 2 |  | -67.67 |
|  | dBm/38.16 MHz | 3 |  | -61.57 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: The noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

#### A.6.6.20.2	Test requirements

If the test is based on PRS, the UE Rx-Tx time difference measurement time fulfils the requirements specified in 9.12.4.1.

The UE shall perform and report the UE Rx-Tx time difference measurements for Cell 1 within the specified UE Rx-Tx time difference measurement time starting from the beginning of time interval T2.

The reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause [10.1.25.3.1].

The UE Rx-Tx time difference measurement time fulfils the UE Rx-Tx measurement accuracy requirements specified in clause [TBD] for Cell 1.

### A.6.6.21	UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA

#### A.6.6.21.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx measurement with TRS for RTT-based PDC meets the requirements specified in clause 9.12.4.2 for measurement delay and clause 10.1.X.2 for measurement accuracy in AWGN propagation condition in FR1 in standalone scenario.

The supported test configurations in listed in Table A.6.6.21.1-1.

Table A.6.6.21.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There is a single cell in the test: PCell (Cell 1) on RF channel 1 in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. Cell 1 does not have TRS transmission during T1 and transmits TRS during T2.

The measurement control information with MeasObjectRxTxDiff set to ‘csi-RS-Ref’ as defined in TS 38.331 [2], shall be provided to the UE during T1. The last TTI containing the RRC message shall be provided to the UE T ms before the start of T2, where T = 10 ms is the maximum processing time of the measurement request.

The UE is configured to transmit SRS during T2.

The general test parameters and cell specific test parameters are as given in Table A.6.6.21.1-2 and Table A.6.6.21.1-3 respectively. The test consists two sub-tests with different TRS BW.

Table A.6.6.21.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell |
| RF Channel Number |  | 1, 2, 3 | 1 |  |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |
|  |  | 2 | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | OFF |  |
| T1 | s | 1, 2, 3 | 1 |  |
| T2 | s | 1, 2, 3 | 1 |  |

Table A.6.6.21.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Values |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1 | N/A |  |
|  |  | 2 | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |
|  |  | 2 | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  |
|  |  | 2 | TRS.1.1 TDD |  |
|  |  | 3 | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  |
| SRS configuration |  | 1 | [PDC-SRS.1] |  |
|  |  | 2 | [PDC-SRS.1] |  |
|  |  | 3 | [PDC-SRS.2] |  |
| Note 2 | dBm/SCS | 1 | -98 |  |
|  |  | 2 | -98 |  |
|  |  | 3 | -95 |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| TRS | dB | 1 | -Infinity | -3 |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| TRS | dB | 1 | -Infinity | -3 |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| TRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -101 |
|  |  | 2 | -Infinity | -101 |
|  |  | 3 | -Infinity | -98 |
| Io | dBm/9.36 MHz | 1 | N/A | -68.28 |
|  | dBm/9.36 MHz | 2 |  | -68.28 |
|  | dBm/38.16 MHz | 3 |  | -62.19 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: TRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

#### A.6.6.21.2	Test requirements

The UE Rx-Tx time difference measurement time fulfils the requirements specified in clause 9.12.4.2. The UE shall be able to report the first Rx-Tx time difference measurement results no later than TBD ms from the beginning of T2.

The UE Rx-Tx time difference measurement time fulfils the UE Rx-Tx measurement accuracy requirements specified in clause 10.1.X.2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%.

## A.6.7	Measurement Performance requirements

Unless explicitly stated otherwise:

-	Reported measurements shall be within defined range of accuracy limits defined in Clause 10 for at least 90 % of the reported cases. If multiple measurement performance requirements are verified in the same test, the reported measurements for each requirement shall be within defined range of accuracy limits of the corresponding requirement defined in Clause 10 for at least 90% of the reported cases.

-	Measurements are performed in RRC_CONNECTED state.

-	The reference channels assume transmission of PDSCH with a maximum number of 5 HARQ transmissions unless otherwise specified.

### A.6.7.1	SS-RSRP

#### A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.2.1.1 and 10.1.2.1.2 for intra-frequency measurements.

##### A.6.7.1.1.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Supported test configurations are shown in table A.6.7.1.1.2-1. Both absolute and relative accuracy of SS-RSRP intra-frequency measurements are tested by using the parameters in A.6.7.1.1.2-2. In all test cases, Cell 1 is the PCell, and Cell 2 is the target cell.

Table A.6.7.1.1.2-1: SS-RSRP  Intra frequency SS-RSRP supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table A.6.7.1.1.2-2: SS-RSRP Intra frequency test parameters

| Parameter |  |  |  |  |  | Unit | Test 1 |  |  | Test 2 |  |  |  |  |  | Test 3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 |  | Cell 2 |  |  |  | Cell 1 |  |  | Cell 2 |  |  |
| Cell ID |  |  |  |  |  |  | 489 | 0 |  | 489 |  | 0 |  |  |  | 489 |  | 0 |  |  |  |
| SSB ARFCN |  |  |  |  |  |  | freq1 |  |  | freq1 |  |  |  |  |  | freq1 |  |  |  |  |  |
| Duplex mode |  |  |  | Config 1 |  |  | FDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 2,3 |  |  | TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TDD configuration |  |  |  | Config 1 |  |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 2 |  |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BWchannel |  |  |  | Config 1 |  | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 2 |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BWP BW |  |  |  | Config 1 |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 2 |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TRS configuration |  |  |  |  | Config 1 |  | TRS.1.1 FDD |  | NA |  | TRS.1.1 FDD |  |  | NA | TRS.1.1 FDD |  |  |  |  |  | NA |
|  |  |  |  |  | Config 2 |  | TRS.1.1 TDD |  | NA |  | TRS.1.1 TDD |  |  | NA | TRS.1.1 TDD |  |  |  |  |  | NA |
|  |  |  |  |  | Config 3 |  | TRS.1.2 TDD |  | NA |  | TRS.1.2 TDD |  |  | NA | TRS.1.2 TDD |  |  |  |  |  | NA |
| DRX Cycle |  |  |  |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  |  | Config 1 |  |  | SR.1.1 FDD |  | - | SR.1.1 FDD |  | - |  |  |  | SR.1.1 FDD |  |  |  |  | - |
|  |  |  |  | Config 2 |  |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |  |  |  | SR.1.1 TDD |  |  |  |  |  |
|  |  |  |  | Config 3 |  |  | SR2.1 TDD |  |  | SR2.1 TDD |  |  |  |  |  | SR2.1 TDD |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  |  | Config 1 |  |  | CR.1.1 FDD |  | - | CR.1.1 FDD |  | - |  |  |  | CR.1.1 FDD |  |  |  |  | - |
|  |  |  |  | Config 2 |  |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |  |  |  | CR.1.1 TDD |  |  |  |  |  |
|  |  |  |  | Config 3 |  |  | CR2.1 TDD |  |  | CR2.1 TDD |  |  |  |  |  | CR2.1 TDD |  |  |  |  |  |
| Control channel RMC |  |  |  | Config 1 |  |  | CCR.1.1 FDD |  | - | CCR.1.1 FDD |  | - |  |  |  | CCR.1.1 FDD |  |  |  |  | - |
|  |  |  |  | Config 2 |  |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |  |  |  | CCR.1.1 TDD |  |  |  |  |  |
|  |  |  |  | Config 3 |  |  | CCR2.1 TDD |  |  | CCR2.1 TDD |  |  |  |  |  | CCR2.1 TDD |  |  |  |  |  |
| SSB configuration |  | Config 1 |  |  |  |  | SSB.1 FR1 |  | SSB.1 FR1 | SSB.1 FR1 |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |
|  |  | Config 2 |  |  |  |  | SSB.1 FR1 |  | SSB.1 FR1 | SSB.1 FR1 |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |
|  |  | Config 3 |  |  |  |  | SSB.2 FR1 |  | SSB.2 FR1 | SSB.2 FR1 |  | SSB.2 FR1 |  |  |  | SSB.2 FR1 |  |  |  | SSB.2 FR1 |  |
| Time offset with Cell 1 |  | Config 1 |  |  |  | ms | - |  | 3 | - |  | 3 |  |  |  | - |  |  |  | 3 |  |
|  |  | Config 2,3 |  |  |  | s | - |  | 3 | - |  | 3 |  |  |  | - |  |  |  | 3 |  |
| SMTC configuration |  | Config 1 |  |  |  |  | SMTC.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  |  |  |  | SMTC.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  |  | Config 1,2 |  | kHz | 15 kHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  |  | 30kHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  | dB | 0 |  | 0 | 0 |  |  | 0 |  |  |  | 0 |  |  |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/15KhZ | -106 |  |  | -88 |  |  |  |  |  |  | -114 |  |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  | -113.5 |  |  |  |  |
|  |  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  | -113 |  |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  | -112.5 |  |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  | -112 |  |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  | -111.5 |  |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  | -111 |  |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  | -110.5 |  |  |  |  |
|  | Config 3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  |  | Not applicableNote 5 |  |  | -94 |  |  |  |  |  |  | -114 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  | -113.5 |  |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  | -113 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  | -112.5 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  | -112 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  | -111.5 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  | -111 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  | -110.5 |  |  |  |  |
| Note2 | Config 1,2 |  |  |  |  | dBm/SCS | -106 |  |  | -88 |  |  |  |  |  |  | Same as Noc/15kHz |  |  |  |  |
|  | Config 3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  |  | Not applicableNote 5 |  |  | -91 |  |  |  |  |  |  | -111 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  | -110.5 |  |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  | -110 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  | -109.5 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  | -109 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  | -108.5 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  | -108 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  | -107.5 |  |  |  |  |
|  |  |  |  |  |  | dB | 2.46 |  | -5.97 | 2.46 |  |  | -5.97 |  |  |  | -0.01 |  |  |  | -4.76 |
|  |  |  |  |  |  | dB | 6 |  | 1 | 6 |  |  | 1 |  |  |  | 3 |  |  |  | 0 |
| SS-RSRPNote3 | Config 1,2 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  |  | dBm/SCS | -100 |  | -105 | -82 |  |  | -87 |  |  |  | -111.00 |  |  |  | -114.00 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  |  | -110.50 |  |  |  | -113.50 |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  |  | -110.00 |  |  |  | -113.00 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  |  | -109.50 |  |  |  | -112.50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  |  | -109.00 |  |  |  | -112.00 |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  |  | -108.50 |  |  |  | -111.50 |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  |  | -108.00 |  |  |  | -111.00 |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  |  | -107.50 |  |  |  | -110.50 |
|  | Config 3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  |  | Not applicableNote 5 |  | Not applicableNote 5 | -85 |  |  | -90 |  |  |  | -108.00 |  |  |  | -111.00 |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  | -107.50 |  |  |  | -110.50 |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  | -107.00 |  |  |  | -110.00 |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  | -106.50 |  |  |  | -109.50 |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  | -106.00 |  |  |  | -109.00 |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  | -105.50 |  |  |  | -108.50 |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  | -105.00 |  |  |  | -108.00 |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  | -104.50 |  |  |  | -107.50 |
| IoNote3 | Config 1,2 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | dBm/ 9.36MHz | -70.09 |  |  | -52.09 |  |  |  |  |  |  | -80.03 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  | -79.53 |  |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  | -79.03 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  | -78.53 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  | -78.03 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  | -77.53 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  | -77.03 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  | -76.53 |  |  |  |  |
|  | Config 3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | dBm/ 38.16MHz | Not applicableNote 5- |  |  | -51.99 |  |  |  |  |  |  | -73.94 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  | -73.44 |  |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  | -72.94 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  | -72.44 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  | -71.94 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  | -71.44 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  | -70.94 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  | -70.44 |  |  |  |  |
| Propagation condition |  |  |  |  |  | - | AWGN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Antenna configuration |  |  |  |  |  |  | 1x2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Subtest 1 is not used when testing with 30kHz SSB SCS. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.7.1.1.3	Test Requirements

The SS-RSRP measurement accuracy for cell 1 and cell 2 shall fulfil absolute requirement in clause 10.1.2.1.1 and relative requirement in clause 10.1.2.1.2.

#### A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.4.1.1 and 10.1.4.1.2 for inter-frequency measurements with the testing configurations for NR cells in Table A.6.7.1.2.1-1.

Table A.6.7.1.2.1-1: Applicable NR configurations for FR1 inter-frequency SS-RSRP accuracy test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.6.7.1.2.2	Test parameters

In this set of test cases there are two cells in the test, PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on a different frequency than the PCell. The test parameters for the Cell 1 and Cell 2 are given in Table A.6.7.1.2.2-1 below. Both absolute and relative accuracy of RSRP inter-frequency measurements are tested by using the parameters in Table A.6.7.1.2.2-1. The inter-frequency measurements are supported by a measurement gap.

Table A.6.7.1.2.2-1: SS-RSRP inter-frequency test parameters

| Parameter |  | Config | Unit | Test 1 |  |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 |  |  | Cell 2 | Cell 1 |  | Cell 2 |
| SSB ARFCN |  | 1~3 |  | freq1 |  |  | freq2 | freq1 |  | freq2 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 |  |  |  | 10: NRB,c = 52 |  |  |
|  |  | 2 |  | 10: NRB,c = 52 |  |  |  | 10: NRB,c = 52 |  |  |
|  |  | 3 |  | 40: NRB,c = 106 |  |  |  | 40: NRB,c = 106 |  |  |
| Duplex mode |  | 1 |  | FDD |  |  |  | FDD |  |  |
|  |  | 2 |  | TDD |  |  |  | TDD |  |  |
|  |  | 3 |  | TDD |  |  |  | TDD |  |  |
| TDD configuration |  | 1 |  | N/A |  |  |  | N/A |  |  |
|  |  | 2 |  | TDDConf.1.1 |  |  |  | TDDConf.1.1 |  |  |
|  |  | 3 |  | TDDConf.2.1 |  |  |  | TDDConf.2.1 |  |  |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD |  |  | - | SR.1.1 FDD |  | - |
|  |  | 2 |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |
|  |  | 3 |  | SR.2.1 FDD |  |  |  | SR.2.1 FDD |  |  |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD |  |  | - | CR.1.1 FDD |  | - |
|  |  | 2 |  | CR.1.1 TDD |  |  | - | CR.1.1 TDD |  | - |
|  |  | 3 |  | CR.2.1 FDD |  |  | - | CR.2.1 FDD |  | - |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD |  |  | - | CCR.1.1 FDD |  | - |
|  |  | 2 |  | CCR.1.1 TDD |  |  | - | CCR.1.1 TDD |  | - |
|  |  | 3 |  | CCR.2.1 TDD |  |  | - | CCR.2.1 TDD |  | - |
| SSB configuration |  | 1 |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |
|  |  | 2 |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |
|  |  | 3 |  | SSB.2 FR1 |  |  |  | SSB.2 FR1 |  |  |
| OCNG Patterns |  | 1~3 |  | OP.1 |  |  |  | OP.1 |  |  |
| TRS configuration |  | 1 |  | TRS.1.1 FDD |  | - |  | TRS.1.1 FDD |  |  |
|  |  | 2 |  | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  |
|  |  | 3 |  | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |
| Time offset with Cell 1 |  | 1 | ms | - | 3 |  |  | - | 3 |  |
|  |  | 2,3 | s | - | 3 |  |  | - | 3 |  |
| SMTC configuration |  | 1 |  | SMTC.2 |  |  |  | SMTC.2 |  |  |
|  |  | 2,3 |  | SMTC.1 |  |  |  | SMTC.1 |  |  |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 |  |  | 0 | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/15kHz | -94.65 |  |  |  | ( for Channel 2 +8dB) |  | -115 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -114.5 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -113.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -113 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  | -112.5 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -112 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -111.5 |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2 | dBm/SSB SCS | -94.65 |  |  |  | ( for Channel 2 +8dB) |  | -115 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -114.5 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -113.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -113 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  | -112.5 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -112 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -111.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3 |  | -91.65 |  |  |  | ( for Channel 2 +8dB) |  | -112.00 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -111.50 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -111.00 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -110.50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -110.00 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  | -109.50 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -109.00 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -108.50 |
|  |  | 1~3 | dB | 10 |  |  | 10 | 13 |  | -3 |
| SS-RSRPNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2 | dBm/SCS | -84.65 |  |  |  | (RSRP for Cell 2 +25dB) |  | -118.00 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -117.50 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -117.00 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -116.50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -116.00 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  | -115.50 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -115.00 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -114.50 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3 |  | -81.65 |  |  |  | (RSRP for Cell 2 +25dB) |  | -115.00 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -114.50 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -114.00 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -113.50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -113.00 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  | -112.50 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -112.00 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -111.50 |
| IoNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2 | dBm/ 9.36MHz | -56.28 |  |  |  | (Io for Channel 2 +19.75dB) |  | -85.28 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -84.78 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -84.28 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -83.78 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -83.28 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  | -82.78 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -82.28 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -81.78 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3 | dBm/ 38.16MHz | -50.19 |  |  |  | (Io for Channel 2 +19.75dB) |  | -79.19 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -78.69 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -78.19 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -77.69 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -77.19 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  | -76.69 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -76.19 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -75.69 |
|  |  | 1~3 | dB | 10 |  |  | 10 | 13 |  | -3 |
| Propagation condition |  | 1~3 | - | AWGN |  |  |  | AWGN |  |  |
| Antenna configuration |  | 1~3 |  | 1x2 |  |  |  | 1x2 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |

##### A.6.7.1.2.3	Test Requirements

The SS-RSRP measurement accuracy for Cell 1 and Cell 2 shall fulfil the absolute requirement in clause 10.1.4.1.1 and relative requirement in clause 10.1.4.1.2.

#### A.6.7.1.3	Void

### A.6.7.2	SS-RSRQ

#### A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.7.1.1.

##### A.6.7.2.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. Supported test configuration are shown in Table A.6.7.2.1.2-1. The absolute accuracy of SS-RSRQ intra-frequency measurement is tested by using the parameters in Table A.6.7.2.1.2-2. In all test cases, Cell 1 is the PCell and Cell 2 is the target cell.

Table A.6.7.2.1.2-1: SS-RSRQ Intra frequency SS-RSRQ supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.2.1.2-2: SS-RSRQ Intra frequency test parameters

| Parameter |  |  |  | Unit | Test 1 |  |  |  |  | Test 2 |  |  |  | Test 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 1 |  |  | Cell 2 |  | Cell 1 |  | Cell 2 |  | Cell 1 |  | Cell 2 |
| SSB ARFCN |  |  |  |  | freq1 |  |  |  |  | freq1 |  |  |  | freq1 |  |  |
| Duplex mode |  |  | Config 1 |  | FDD |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |  |  |  |  |
| TDD configuration |  |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |  |  |
| BWchannel |  |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |  |
| Gap Pattern ID |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |
| BWP configuration |  |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |
| DRX Cycle |  |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 |  | SR.1.1 FDD |  | - |  |  | SR.1.1 FDD |  | - |  | SR.1.1 FDD |  | - |
|  |  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |
|  |  |  | Config 3 |  | SR2.1 TDD |  |  |  |  | SR2.1 TDD |  |  |  | SR2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 |  | CR.1.1 FDD |  | - |  |  | CR.1.1 FDD |  | - |  | CR.1.1 FDD |  |  |
|  |  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |
|  |  |  | Config 3 |  | CR.2.1 TDD |  |  |  |  | CR.2.1 TDD |  |  |  | CR.2.1 TDD |  |  |
| Control Channel RMC |  |  | Config 1 |  | CCR.1.1 FDD |  | - |  |  | CCR.1.1 FDD |  | - |  | CCR.1.1 FDD |  | - |
|  |  |  | Config 2 |  | CCR.1.1 TDD |  |  |  |  | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |  |  |
|  |  |  | Config 3 |  | CCR.2.1 TDD |  |  |  |  | CCR.2.1 TDD |  |  |  | CCR.2.1 TDD |  |  |
| TRS Configuration |  |  | Config 1 |  | TRS.1.1 FDD |  | - |  |  | TRS.1.1 FDD |  | - |  | TRS.1.1 FDD |  | - |
|  |  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  |
|  |  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |
| OCNG Patterns |  |  |  |  | OP. 1 |  |  |  |  |  |  |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  |  | Config 1 | ms | - | 3 |  |  | - |  | 3 |  | - |  | 3 |  |
|  |  |  | Config 2,3 | s | - | 3 |  |  | - |  | 3 |  | - |  | 3 |  |
| SMTC configuration |  |  | Config 1 |  | SMTC.2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |  |  |  |  |  |  |
| SSB configuration |  |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |  |  |  |  |
| CSI-RS for tracking |  |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | 30kHz |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |  | 0 |  |  | 0 |  | 0 |  | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -85 |  |  |  |  | -101 |  |  |  | -114 |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  | -113.5 |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  | -113 |  |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  | -112.5 |  |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  | -112 |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  | -111.5 |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  | -111 |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  | -110.5 |  |  |
|  | Config 3 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -91 |  |  |  |  | - |  |  |  | -114 |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  | -113.5 |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  | -113 |  |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  | -112.5 |  |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  | -112 |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  | -111.5 |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  | -111 |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  | -110.5 |  |  |
| Note2 | Config 1,2 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/SCS | -85 |  |  |  |  | -101 |  |  |  | -114 -113.5 -113 -112.5 -112 -111.5 -111 -110.5 |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Config 3 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -88 |  |  |  |  | - |  |  |  | -111 |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  | -110.5 |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  | -110 |  |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  | -109.5 |  |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  | -109 |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  | -108.5 |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  | -108 |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  | -107.5 |  |  |
|  |  |  |  | dB | -1.76 |  |  |  |  | -4.7 |  |  |  | -5..46 |  | -5.46 |
|  |  |  |  | dB | 3 |  | 3 |  |  | -2.9 |  | -2.9 |  | -4 |  | -4 |
| SS-RSRPNote3 | Config 1,2 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/SCS | -82 |  | -82 |  |  | -103.9 |  | -103.9 |  | -118 |  | -118 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  | -117.5 |  | -117.5 |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  | -117 |  | -117 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  | -116.5 |  | -116.5 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  | -116 |  | -116 |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  | -115.5 |  | -115.5 |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  | -115 |  | -115 |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  | -114.5 |  | -114.5 |
|  | Config 3 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -85 |  | -85 |  |  | - |  | - |  | -115 |  | -115 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  | -114.5 |  | -114.5 |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  | -114 |  | -114 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  | -113.5 |  | -113.5 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  | -113 |  | -113 |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  | -112.5 |  | -112.5 |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  | -112 |  | -112 |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  | -111.5 |  | -111.5 |
| SS-RSRQ Note3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dB | -14.77 |  | -14.77 |  |  | -16.76 |  | -16.76 |  | -17.34 |  | -17.34 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IoNote3 |  | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 9.36MHz | -50 |  |  |  |  | -70 |  |  |  | -83.5 |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  | -83 |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  | -82.5 |  |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  | -82 |  |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  | -81.5 |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  | -81 |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  | -80.5 |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  | -80 |  |  |
|  |  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 38.16MHz | -50 |  |  |  |  | - |  |  |  | -77.4 |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  | -76.9 |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  | -76.4 |  |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  | -75.9 |  |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  | -75.4 |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  | -74.9 |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  | -74.4 |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  | -73.9 |  |  |
| Propagation condition |  |  |  | - | AWGN |  | AWGN |  |  | AWGN |  | AWGN |  | AWGN |  | AWGN |
| Antenna configuration |  |  |  |  | 1x2 |  | 1x2 |  |  | 1x2 |  | 1x2 |  | 1x2 |  | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.7.2.1.3	Test Requirements

The SS-RSRQ measurement accuracy shall fulfil the requirements in clause 10.1.7.1.1.

#### A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.9.1.1 and 10.1.9.1.2.

##### A.6.7.2.2.2	Test Parameters

In this test case the two cells (i.e., Cell 1 and Cell 2) are on different carrier frequencies and measurement gaps are provided. Supported test configurations are shown in Table A.6.7.2.2.2-1. Both absolute accuracy and relative accuracy requirements of SS-RSRQ inter-frequency measurement are tested by using test parameters in Table A.6.7.2.2.2-2. In all test cases, Cell 1 is the PCell and Cell 2 is target cell.

Table A.6.7.2.2.2-1: SS-RSRQ Inter frequency SS-RSRQ supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.2.2.2-2: SS-RSRQ Inter frequency test parameters

| Parameter |  |  |  | Unit | Test 1 |  |  |  | Test 2 |  |  |  |  |  | Test 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 1 |  | Cell 2 |  | Cell 1 |  |  | Cell 2 |  |  | Cell 1 |  |  | Cell 2 |  |
| SSB ARFCN |  |  |  |  | freq1 | freq2 |  |  | freq1 |  |  |  | freq2 |  | freq1 |  | freq2 |  |  |
| Duplex mode |  | Config 1 |  |  | FDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  |  | TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BWchannel |  | Config 1 |  | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Gap pattern ID |  | Config 1,2,3 |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BWP BW |  | Config 1 |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DRX Cycle |  |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  |  | SR.1.1 FDD | - |  |  | SR.1.1 FDD |  |  | - |  |  | SR.1.1 FDD |  |  | - |  |
|  |  | Config 2,5 |  |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |  |  |  | SR.1.1 TDD |  |  |  |  |
|  |  | Config 3,6 |  |  | SR2.1 TDD |  |  |  | SR2.1 TDD |  |  |  |  |  | SR2.1 TDD |  |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  |  | CR.1.1 FDD | - |  |  | R.1.1 FDD |  |  | - |  |  | CR.1.1 FDD |  |  |  |  |
|  |  | Config 2 |  |  | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |  |  |  | CR.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  |  | CR2.1 TDD |  |  |  | CR2.1 TDD |  |  |  |  |  | CR2.1 TDD |  |  |  |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  |  | CCR.1.1 FDD | - |  |  | CCR.1.1 FDD |  |  | - |  |  | CCR.1.1 FDD |  |  | - |  |
|  |  | Config 2 |  |  | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |  |  |  |  |  | CCR.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  |  | CCR2.1 TDD |  |  |  | CCR2.1 TDD |  |  |  |  |  | CCR2.1 TDD |  |  |  |  |
| TRS Configuration |  | Config 1 |  |  | TRS.1.1 FDD | - |  |  | TRS.1.1 FDD |  |  | - |  |  | TRS.1.1 FDD |  |  | - |  |
|  |  | Config 2 |  |  | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  |  |  |  | TRS.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  |  | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |  |  |  | TRS.1.2 TDD |  |  |  |  |
| OCNG Patterns |  |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 1 |  | ms | - |  |  | 3 |  | - | 3 |  |  | - |  | 3 |  |  |  |
|  |  | Config 2,3 |  | s | - |  |  | 3 |  | - | 3 |  |  | - |  | 3 |  |  |  |
| SMTC configuration |  | Config 1 |  |  | SMTC pattern 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  |  | SMTC pattern 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSB configuration |  | Config 1,2 |  |  | SSB pattern 1 in FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | SSB pattern 2 in FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CSI-RS for tracking |  | Config 1 |  |  | TRS.1.1 FDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  |  | TRS.1.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | TRS.1.2 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 |  | kHz | 15 kHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | 30 kHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -80.18 |  |  |  | -106 |  |  |  |  |  | -116 |  |  |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  | -115.5 |  |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  | -115 |  |  |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  | -114.5 |  |  |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  | -114 |  |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  | -113.5 |  |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  | -113 |  |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  | -112.5 |  |  |  |  |
| Note2 | Config 3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -86.27 |  |  |  | -113 |  |  |  |  |  | -116 |  |  |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  | -115.5 |  |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  | -115 |  |  |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  | -114.5 |  |  |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  | -114 |  |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  | -113.5 |  |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  | -113 |  |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  | -112.5 |  |  |  |  |
| Note2 | Config 1,2 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -80.18 |  |  |  | -106 |  |  |  |  |  | -116 |  |  |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  | -115.5 |  |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  | -115 |  |  |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  | -114.5 |  |  |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  | -114 |  |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  | -113.5 |  |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  | -113 |  |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  | -112.5 |  |  |  |  |
|  | Config 3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -83.27 |  |  |  | -110 |  |  |  |  |  | -113 |  |  |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  | -112.5 |  |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  | -112 |  |  |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  | -111.5 |  |  |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  | -111 |  |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  | -110.5 |  |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  | -110 |  |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  | -109.5 |  |  |  |  |
|  |  |  |  | dB | -1.75 |  |  |  | -1.75 |  |  |  |  |  | 3 |  |  | -1.75 |  |
|  |  |  |  | dB | -1.75 |  |  |  | -1.75 |  |  |  |  |  | 3 |  |  | -1.75 |  |
| SS-RSRPNote3 | Config 1,2 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/SCS | -81.93 | -81.93 |  |  | -107.75 |  |  | -107.75 |  |  | -113 |  |  |  | -117.75 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  | -112.5 |  |  |  | -117.25 |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  | -112 |  |  |  | -116.75 |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  | -111.5 |  |  |  | -116.25 |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  | -111 |  |  |  | -115.75 |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  | -110.5 |  |  |  | -115.2 |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  | -110 |  |  |  | -114.75 |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  | -109.5 |  |  |  | -114.25 |
|  | Config 3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -85.02 | -85.02 |  |  | -111.75 |  |  | -111.75 |  |  | -110 |  |  |  | -114.75 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  | -109.5 |  |  |  | -114.25 |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  | -109 |  |  |  | -113.75 |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  | -108.5 |  |  |  | -113.25 |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  | -108 |  |  |  | -112.75 |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  | -107.5 |  |  |  | -112.2 |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  | -107 |  |  |  | -111.75 |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  | -106.5 |  |  |  | -111.25 |
| SS-RSRQNote3 |  |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dB | -14.77 | -14.77 |  |  | -40.59 |  |  | -40.59 |  |  | 12.56T |  |  |  | 14.76T |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IoNote3 | Config 1,2 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/SCS | -50 |  |  |  | -75.83 |  |  |  |  |  | -83.28 |  |  |  | -85.83 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  | -82.78 |  |  |  | -85.33 |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  | -82.28 |  |  |  | -84.83 |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  | -81.78 |  |  |  | -84.33 |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  | -81.28 |  |  |  | -83.83 |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  | -80.78 |  |  |  | -83.33 |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  | -80.28 |  |  |  | -82.83 |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  | -79.78 |  |  |  | -82.33 |
|  | Config 3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -50 |  |  |  | -76.73 |  |  |  |  |  | -77.19 |  |  |  | -79.73 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  | -76.69 |  |  |  | -79.23 |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  | -76.19 |  |  |  | -78.73 |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  | -75.69 |  |  |  | -78.23 |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  | -75.19 |  |  |  | -77.73 |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  | -74.69 |  |  |  | -77.23 |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  | -74.19 |  |  |  | -76.73 |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  | -73.69 |  |  |  | -76.53 |
| Propagation condition |  |  |  | - | AWGN | AWGN |  |  | AWGN |  |  | AWGN |  |  | AWGN |  |  | AWGN |  |
| Antenna configuration |  |  |  |  | 1x2 | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.7.2.2.3	Test Requirements

The SS-RSRQ measurement accuracy shall fulfil the requirements in clause 10.1.9.1.1 and 10.1.9.1.2.

### A.6.7.3	SS-SINR

#### A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.3.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.12.1.1.

##### A.6.7.3.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. Supported test configuration are shown in Table A.6.7.3.1.2-1. The absolute accuracy of SS-SINR intra-frequency measurement is tested by using the parameters in Table A.6.7.3.1.2-2. In all test cases, Cell 1 is the PCell and Cell 2 is the target cell.

Table A.6.7.3.1.2-1: SS-SINR Intra frequency SS-SINR supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.3.1.2-2: SS-SINR Intra frequency test parameters

| Parameter |  |  |  | Unit | Test 1 |  |  | Test 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 |  |  | Cell 2 |
| SSB ARFCN |  |  |  |  | freq1 |  |  | freq1 |  |  |  |
| Duplex mode |  |  | Config 1 |  | FDD |  |  |  |  |  |  |
|  |  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |
| TDD configuration |  |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |
|  |  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |
| DRX Cycle configuration |  |  |  | ms | Not Applicable |  |  |  |  |  |  |
| TRS configuration |  |  | Config 1 |  | TRS.1.1 FDD |  |  | TRS.1.1 FDD |  |  |  |
|  |  |  | Config 2 |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |  |
|  |  |  | Config 3 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 |  | SR.1.1 FDD | - |  | SR.1.1 FDD |  |  | - |
|  |  |  | Config 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 |  | SR.2.1 TDD |  |  | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 |  | CR.1.1 FDD | - |  | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2 |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 |  | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD |  |  | - |
|  |  |  | Config 2 |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 |  | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  |  | Not Applicable |  |  |  |  |  |  |
| SMTC configuration |  |  | Config 1 |  | SMTC.2 |  |  |  |  |  |  |
|  |  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |  |
| Time offset with Cell 1 |  |  | Config 1 | ms | - |  | 3 |  | - | 3 |  |
|  |  |  | Config 2,3 | ms | - |  | 3 |  | - | 3 |  |
| SSB configuration |  |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |
|  |  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1,2 | kHz | 15 |  |  |  |  |  |  |
|  |  |  | Config 3 |  | 30 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 | 0 |  | 0 |  |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -93 |  |  | -116 |  |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  | -115.5 |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  | -115 |  |  |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  | -114.5 |  |  |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  | -114 |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  | -113.5 |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  | -113 |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  | -112.5 |  |  |  |
| Note2 | Config 1,2 |  |  | dBm/SCS | -93 |  |  | Same as Noc for 15 kHz |  |  |  |
|  | Config 3 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -90 |  |  | -113 |  |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  | -112.5 |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  | -112 |  |  |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  | -111.5 |  |  |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  | -111 |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  | -110.5 |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  | -110 |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  | -109.5 |  |  |  |
|  |  |  |  | dB | 0 | -3.19 |  | -5.46 |  |  | -5.46 |
|  |  |  |  | dB | 4.54 | 2.66 |  | -4 |  |  | -4 |
| SS-RSRPNote3 | Config 1,2 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/SCS | -88.46 | -90.34 |  | -120 |  |  | -120 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  | -119.5 |  |  | -119.5 |
|  |  |  | NR_TDD_FR1_C |  |  |  |  | -119 |  |  | -119 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  | -118.5 |  |  | -118.5 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  | -118 |  |  | -118 |
|  |  |  | NR_FDD_FR1_F |  |  |  |  | -117.5 |  |  | -117.5 |
|  |  |  | NR_FDD_FR1_G |  |  |  |  | -117 |  |  | -117 |
|  |  |  | NR_FDD_FR1_H |  |  |  |  | -116.5 |  |  | -116.5 |
|  | Config 3 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -85.46 | -87.34 |  | -117 |  |  | -117 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  | -116.5 |  |  | -116.5 |
|  |  |  | NR_TDD_FR1_C |  |  |  |  | -116 |  |  | -116 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  | -115.5 |  |  | -115.5 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  | -115 |  |  | -115 |
|  |  |  | NR_FDD_FR1_F |  |  |  |  | -114.5 |  |  | -114.5 |
|  |  |  | NR_FDD_FR1_G |  |  |  |  | -114 |  |  | -114 |
|  |  |  | NR_FDD_FR1_H |  |  |  |  | -113.5 |  |  | -113.5 |
| SS-SINR Note3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dB | 0 | -3.19 |  | -5.46 |  |  | -5.46 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |
| IoNote3 |  | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 9.36MHz | -57.5 |  |  | -85.51 |  |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  | -85.01 |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  | -84.51 |  |  |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  | -84.01 |  |  |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  | -83.51 |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  | -83.01 |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  | -82.51 |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  | -82.01 |  |  |  |
|  |  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 38.16MHz | -51.41 |  |  | -79.41 |  |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  | -78.91 |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  | -78.41 |  |  |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  | -77.91 |  |  |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  | -77.41 |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  | -76.91 |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  | -76.41 |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  | -75.91 |  |  |  |
| Propagation condition |  |  |  | - | AWGN |  |  |  |  |  |  |
| Antenna configuration |  |  |  | - | 1x2 |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.7.3.1.3	Test Requirements

The SS-SINR measurement accuracy shall fulfil the requirements in clause 10.1.12.1.1.

#### A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.3.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.14.1.1 and 10.1.14.1.2.

##### A.6.7.3.2.2	Test Parameters

In this test case the two cells (i.e., Cell 1 and Cell 2) are on different carrier frequencies and measurement gaps are provided. Supported test configurations are shown in Table A.6.7.3.2.2-1. Both absolute accuracy and relative accuracy requirements of SS-SINR inter-frequency measurement are tested by using test parameters in Table A.6.7.3.2.2-2. In all test cases, Cell 1 is the PCell and Cell 2 is target cell.

Table A.6.7.3.2.2-1: SS-SINR Inter frequency SS-SINR supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.3.2.2-2: SS-SINR Inter frequency test parameters

| Parameter |  |  |  |  | Unit | Test 1 |  |  | Test 2 |  |  |  |  | Test 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 |  |  | Cell 2 |  | Cell 1 |  |  | Cell 2 |
| SSB ARFCN |  |  |  |  |  | freq1 | freq2 |  | freq1 |  |  | freq2 |  | freq1 |  | freq2 |  |
| Duplex mode |  |  | Config 1 |  |  | FDD |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2,3 |  |  | TDD |  |  |  |  |  |  |  |  |  |  |  |
| TDD configuration |  |  | Config 1 |  |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2 |  |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |
| DRX Cycle configuration |  |  |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |
| Gap pattern ID |  |  |  |  |  | 0 |  | - |  | 0 | - |  | 0 |  | - |  |  |
| TRS configuration |  |  | Config 1 |  |  | TRS.1.1 FDD |  |  | TRS.1.1 FDD |  |  |  |  | TRS.1.1 FDD |  |  |  |
|  |  |  | Config 2 |  |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |  |  | TRS.1.1 TDD |  |  |  |
|  |  |  | Config 3 |  |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |  |  | TRS.1.2 TDD |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 |  |  | SR.1.1 FDD | - |  | SR.1.1 FDD |  |  | - |  | SR.1.1 FDD |  |  | - |
|  |  |  | Config 2 |  |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |  |  | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 |  |  | SR2.1 TDD |  |  | SR2.1 TDD |  |  |  |  | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 |  |  | CR.1.1 FDD | - |  | CR.1.1 FDD |  |  | - |  | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2 |  |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |  |  | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 |  |  | CR2.1 TDD |  |  | CR2.1 TDD |  |  |  |  | CR2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 |  |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD |  |  | - |  | CCR.1.1 FDD |  |  | - |
|  |  |  | Config 2 |  |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |  |  | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 |  |  | CCR2.1 TDD |  |  | CCR2.1 TDD |  |  |  |  | CCR2.1 TDD |  |  |  |
| OCNG Patterns |  |  |  |  |  | OP.1 |  |  |  |  |  |  |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  |  |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  |  | Config 1 |  | ms | - |  | 3 |  | - | 3 |  | - |  | 3 |  |  |
|  |  |  | Config 2,3 |  | s | - |  | 3 |  | - | 3 |  | - |  | 3 |  |  |
| SMTC configuration |  |  | Config 1 |  |  | SMTC pattern 2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2,3 |  |  | SMTC pattern 1 |  |  |  |  |  |  |  |  |  |  |  |
| SSB configuration |  |  | Config 1,2 |  |  | SSB.1 FR1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  |  | SSB.2 FR1 |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1,2 |  | kHz | 15 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  | dB | 0 | 0 |  | 0 |  |  | 0 |  | 0 |  |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 |  |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -88 |  |  | -108.5 |  |  |  |  | -119.5 |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  | -119 |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  | -118.5 |  |  |  |
|  |  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  | -118 |  |  |  |
|  |  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  | -117.5 |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  | -117 |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  | -116.5 |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  | -116 |  |  |  |
| Note2 | Config 1,2 |  |  |  | dBm/SCS | -88 |  |  | -108.5 |  |  |  |  | Same as Noc for 15kHz |  |  |  |
|  | Config 3 |  |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -85 |  |  | -105.5 |  |  |  |  | -116.5 |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  | -116 |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  | -115.5 |  |  |  |
|  |  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  | -115 |  |  |  |
|  |  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  | -114.5 |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  | -114 |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  | -114.5 |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  | -113 |  |  |  |
|  |  |  |  |  | dB | -1.75 | -1.75 |  | 20 |  |  | 20 |  | -4.0 |  |  | -4.0 |
|  |  |  |  |  | dB | -1.75 |  |  | 20 |  |  |  |  | -4.0 |  |  |  |
| SS-RSRP Note3 | Config 1,2 |  |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/SCS | -89.75 |  |  | -88.5 |  |  |  |  | -123.5 |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  | -123 |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  | -122.5 |  |  |  |
|  |  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  | -122 |  |  |  |
|  |  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  | -121.5 |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  | -121 |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  | -120.5 |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  | -120 |  |  |  |
|  | Config 3 |  |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -86.75 |  |  | -85.5 |  |  |  |  | -120.5 |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  | -120 |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  | -119.5 |  |  |  |
|  |  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  | -119 |  |  |  |
|  |  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  | -118.5 |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  | -118 |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  | -117.5 |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  | -117 |  |  |  |
| SS-SINRNote3 |  |  |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dB | -1.75 |  |  | 20 |  |  |  |  | -4.0 |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IoNote3 | Config 1,2 |  |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/ 9.36MHz | -57.83 |  |  | -60.5 |  |  |  |  | -90.09 |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  | -89.59 |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  | -89.09 |  |  |  |
|  |  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  | -88.59 |  |  |  |
|  |  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  | -88.09 |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  | -87.59 |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  | -87.09 |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  | -86.59 |  |  |  |
|  | Config 3 |  |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/ 38.16MHz | -51.73 |  |  | -54.41 |  |  |  |  | -84 |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  | -83.5 |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  | -83 |  |  |  |
|  |  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  | -82.5 |  |  |  |
|  |  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  | -82 |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  | -81.5 |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  | -81 |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  | -80.5 |  |  |  |
| Propagation condition |  |  |  |  | - | AWGN |  |  |  |  |  |  |  |  |  |  |  |
| Antenna configuration |  |  |  |  | - | 1x2 |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.7.3.2.3	Test Requirements

The SS-SINR measurement accuracy shall fulfil the requirements in clause 10.1.14.1.1 and 10.1.14.1.2.

### A.6.7.4	L1-RSRP measurement for beam reporting

#### A.6.7.4.1	SSB based L1-RSRP measurement

##### A.6.7.4.1.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clause 9.5.2 and clause 10.1.19.1 for L1-RSRP measurements based on SSB with the testing configurations for NR cells in Table A.6.7.4.1.1-1.

Table A.6.7.4.1.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.6.7.4.1.2	Test parameters

In this set of test cases there one cell in the test, PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.6.7.4.1.2-1 below. The absolute and relative accuracy of L1-RSRP measurements are tested by using the parameters in Table A.6.7.4.1.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured one SSB resource set with two SSB resources. UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSB resources 0 and 1.

Table A.6.7.4.1.2-1: FR1 SSB based L1-RSRP test parameters

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1~3 |  | freq1 | freq1 |
| Duplex mode |  | 1 |  | FDD | FDD |
|  |  | 2 |  | TDD | TDD |
|  |  | 3 |  | TDD | TDD |
| TDD Configuration |  | 1 |  | N/A | N/A |
|  |  | 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 2 |  | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 3 |  | 40: NRB,c = 106 | 40: NRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  |  | 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 3 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  |  | 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 3 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 3 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.3 FR1 | SSB.3 FR1 |
|  |  | 2 |  | SSB.3 FR1 | SSB.3 FR1 |
|  |  | 3 |  | SSB.4 FR1 | SSB.4 FR1 |
| OCNG Patterns |  | 1~3 |  | OP.1 | OP.1 |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  |  | 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 3 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~3 |  | SMTC.1 | SMTC.1 |
| reportConfigType |  | 1~3 |  | periodic | periodic |
| reportQuantity |  | 1~3 |  | ssb-Index-RSRP | ssb-Index-RSRP |
| Number of reported RS |  | 1~3 |  | 2 | 2 |
| L1-RSRP reporting period |  | 1~3 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/15kHz | -94.65 | -117 |
|  | NR_FDD_FR1_B |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/SSB SCS | -94.65 | -117 |
|  | NR_FDD_FR1_B |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -91.65 | -114 |
|  | NR_FDD_FR1_B |  |  |  | -113.5 |
|  | NR_TDD_FR1_C |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -112.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -112 |
|  | NR_FDD_FR1_F |  |  |  | -111.5 |
|  | NR_FDD_FR1_G |  |  |  | -111 |
|  | NR_FDD_FR1_H |  |  |  | -110.5 |
|  |  | 1~3 | dB | 10 | -3 |
| SSB RSRP Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/SSB SCS | -84.65 | -120 |
|  | NR_FDD_FR1_B |  |  |  | -119.5 |
|  | NR_TDD_FR1_C |  |  |  | -119 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -118.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -118 |
|  | NR_FDD_FR1_F |  |  |  | -117.5 |
|  | NR_FDD_FR1_G |  |  |  | -117 |
|  | NR_FDD_FR1_H |  |  |  | -116.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -81.65 | -117 |
|  | NR_FDD_FR1_B |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
| Io Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/9.36 MHz | -56.28 | -87.28 |
|  | NR_FDD_FR1_B |  |  |  | -86.78 |
|  | NR_TDD_FR1_C |  |  |  | -86.28 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -85.78 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -85.28 |
|  | NR_FDD_FR1_F |  |  |  | -84.78 |
|  | NR_FDD_FR1_G |  |  |  | -84.28 |
|  | NR_FDD_FR1_H |  |  |  | -83.78 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 | dBm/38.16 MHz | -50.19 | -81.19 |
|  | NR_FDD_FR1_B |  |  |  | -80.69 |
|  | NR_TDD_FR1_C |  |  |  | -80.19 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -79.69 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -79.19 |
|  | NR_FDD_FR1_F |  |  |  | -78.69 |
|  | NR_FDD_FR1_G |  |  |  | -78.19 |
|  | NR_FDD_FR1_H |  |  |  | -77.69 |
|  |  | 1~3 | dB | 10 | -3 |
| Propagation condition |  | 1~3 |  | AWGN | AWGN |
| Antenna configuration |  | 1~3 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |

##### A.6.7.4.1.3	Test Requirements

The L1-RSRP measurement accuracy for SSB resource reported by UE in L1-RSRP report (SSB#0 or SSB#1) of Cell 2 shall fulfil the requirements in clauses 10.1.19.1.

#### A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off

##### A.6.7.4.2.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clause 9.5.3 and clause 10.1.19.2 for L1-RSRP measurements based on CSI-RS with the testing configurations for NR cells in Table A.6.7.4.2.1-1.

Table A.6.7.4.2.1-1: Applicable NR configurations for FR1 CSI-RS based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.6.7.4.2.2	Test parameters

In this set of test cases there are one cell in the test, PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.6.7.4.2.2-1 below. The absolute and relative accuracy of L1-RSRP measurements are tested by using the parameters in Table A.6.7.4.2.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured one CSI-RS resource set with two CSI-RS resources. UE is configured to perform RLM and BFD based on SSB 0 and 1. CSI-RS is not transmitted in the same OFDM symbols as SSB.

Table A.6.7.4.2.2-1: FR1 CSI-RS based L1-RSRP test parameters

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1~3 |  | freq1 | freq1 |
| Duplex mode |  | 1 |  | FDD | FDD |
|  |  | 2 |  | TDD | TDD |
|  |  | 3 |  | TDD | TDD |
| TDD Configuration |  | 1 |  | N/A | N/A |
|  |  | 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 2 |  | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 3 |  | 40: NRB,c = 106 | 40: NRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  |  | 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 3 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  |  | 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 3 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 3 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.3 FR1 | SSB.3 FR1 |
|  |  | 2 |  | SSB.3 FR1 | SSB.3 FR1 |
|  |  | 3 |  | SSB.4 FR1 | SSB.4 FR1 |
| OCNG Patterns |  | 1~3 |  | OP.1 | OP.1 |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  |  | 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 3 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~3 |  | SMTC.1 | SMTC.1 |
| CSI-RS |  | 1 |  | CSI-RS 1.2 FDD | CSI-RS 1.2 FDD |
|  |  | 2 |  | CSI-RS 1.2 TDD | CSI-RS 1.2 TDD |
|  |  | 3 |  | CSI-RS 2.2 TDD | CSI-RS 2.2 FDD |
| reportConfigType |  | 1~3 |  | periodic | periodic |
| reportQuantity |  | 1~3 |  | cri-RSRP | cri-RSRP |
| Number of reported RS |  | 1~3 |  | 2 | 2 |
| L1-RSRP reporting period |  | 1~3 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/15kHz | -94.65 | -117 |
|  | NR_FDD_FR1_B |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/CSI-RS SCS | -94.65 | -117 |
|  | NR_FDD_FR1_B |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -91.65 | -114 |
|  | NR_FDD_FR1_B |  |  |  | -113.5 |
|  | NR_TDD_FR1_C |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -112.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -112 |
|  | NR_FDD_FR1_F |  |  |  | -111.5 |
|  | NR_FDD_FR1_G |  |  |  | -111 |
|  | NR_FDD_FR1_H |  |  |  | -110.5 |
|  |  | 1~3 | dB | 10 | -3 |
| CSI-RS RSRP Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/CSI-RS SCS | -84.65 | -120 |
|  | NR_FDD_FR1_B |  |  |  | -119.5 |
|  | NR_TDD_FR1_C |  |  |  | -119 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -118.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -118 |
|  | NR_FDD_FR1_F |  |  |  | -117.5 |
|  | NR_FDD_FR1_G |  |  |  | -117 |
|  | NR_FDD_FR1_H |  |  |  | -116.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -81.65 | -117 |
|  | NR_FDD_FR1_B |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
| Io Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/9.36 MHz | -56.28 | -87.28 |
|  | NR_FDD_FR1_B |  |  |  | -86.78 |
|  | NR_TDD_FR1_C |  |  |  | -86.28 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -85.78 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -85.28 |
|  | NR_FDD_FR1_F |  |  |  | -84.78 |
|  | NR_FDD_FR1_G |  |  |  | -84.28 |
|  | NR_FDD_FR1_H |  |  |  | -83.78 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 | dBm/38.16 MHz | -50.19 | -81.19 |
|  | NR_FDD_FR1_B |  |  |  | -80.69 |
|  | NR_TDD_FR1_C |  |  |  | -80.19 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -79.69 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -79.19 |
|  | NR_FDD_FR1_F |  |  |  | -78.69 |
|  | NR_FDD_FR1_G |  |  |  | -78.19 |
|  | NR_FDD_FR1_H |  |  |  | -77.69 |
|  |  | 1~3 | dB | 10 | -3 |
| Propagation condition |  | 1~3 |  | AWGN | AWGN |
| Antenna configuration |  | 1~3 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |

##### A.6.7.4.2.3	Test Requirements

The L1-RSRP measurement accuracy for CSI-RS resource reported by UE in L1-RSRP report (CSI-RS#0 or CSI-RS#1) of Cell 1 shall fulfil the requirements in clause 10.1.19.2.

### A.6.7.5	E-UTRAN RSRP

#### A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell

##### A.6.7.5.1.1	Test Purpose and Environment

The purpose of this test is to verify that the E-UTRAN RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.2.2 for SA inter-RAT E-UTRAN RSRP measurements.

##### A.6.7.5.1.2	Test parameters

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an E-UTRAN inter-RAT neighbour cell. Supported test configurations are shown in table A.6.7.5.1.2-1. The measurement accuracy of SA inter-RAT E-UTRAN RSRP are tested by using the parameters in A.6.7.5.1.2-2 and A.6.7.5.1.2-3.

Table A.6.7.5.1.2-1: Inter-RAT E-UTRAN RSRP supported test configurations with FR1 serving cell

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, E-UTRAN FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, E-UTRAN FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, E-UTRAN FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, E-UTRAN TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, E-UTRAN TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, E-UTRAN TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.5.1.2-2: NR Cell specific test parameters for SA Inter-RAT E-UTRAN RSRP test parameters

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| NR RF channel number |  |  | 1 |
| Duplex mode | Config 1, 4 |  | FDD |
|  | Config 2, 3, 5, 6 |  | TDD |
| TDD Configuration | Config 1, 4 |  | N/A |
|  | Config 2, 5 |  | TDDConf.1.1 |
|  | Config 3, 6 |  | TDDConf.2.1 |
| BWchannel | Config 1, 4 | MHz | 10: NRB,c = 52 (FDD) |
|  | Config 2, 5 |  | 10: NRB,c = 52 (TDD) |
|  | Config 3, 6 |  | 40: NRB,c = 106 (TDD) |
| Gap pattern Id |  |  | 0 |
| PDSCH reference measurement channel | Config 1, 4 |  | SR.1.1 FDD |
|  | Config 2, 5 |  | SR.1.1 TDD |
|  | Config 3, 6 |  | SR.2.1 TDD |
| RMSI CORSET reference channel | Config 1, 4 |  | CR.1.1 FDD |
|  | Config 2, 5 |  | CR.1.1 TDD |
|  | Config 3, 6 |  | CR.2.1 TDD |
| Dedicated CORSET reference channel | Config 1, 4 |  | CCR.1.1 FDD |
|  | Config 2, 5 |  | CCR.1.1 TDD |
|  | Config 3, 6 |  | CCR.2.1 TDD |
| CSI-RS for tracking | Config 1, 4 |  | TRS.1.1 FDD |
|  | Config 2, 5 |  | TRS.1.1 TDD |
|  | Config 3, 6 |  | TRS.1.2 TDD |
| BWP configurations | Initial DL BWP |  | DLBWP.0.1 |
|  | Dedicated DL BWP |  | DLBWP.1.1 |
|  | Initial UL BWP |  | ULBWP.0.1 |
|  | Dedicated UL BWP |  | ULBWP.1.1 |
| OCNG patternNote1 |  |  | OP.1 |
| SMTC configuration |  |  | SMTC.1 |
| SSB configuration | Config 1, 2, 4, 5 |  | SSB.1 FR1 |
|  | Config 3, 6 |  | SSB.2 FR1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |
| NocNote2 |  | dBm/15 kHz | -104 |
| NocNote2 | Config 1, 2, 4, 5 | dBm/SCS | -104 |
|  | Config 3, 6 |  | -101 |
| Ês/Noc |  | dB | 17 |
| Ês/IotNote3 |  | dB | 17 |
| SS-RSRPNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| SSB_RPNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| IoNote3 | Config 1, 2, 4, 5 | dBm/9.36 MHz | -58.96 |
|  | Config 3, 6 | dBm/38.16 MHz | -52.87 |
| Propagation condition |  |  | AWGN |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table A.6.7.5.1.2-3: E-UTRAN Cell specific test parameters for SA Inter-RAT E-UTRAN RSRP test parameters

| Parameter |  | Unit | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |
| E-UTRA RF channel number |  |  | 1 |  |
| Duplex mode | Config 1, 2, 3 |  | FDD |  |
|  | Config 4, 5, 6 |  | TDD |  |
| TDD special subframe configurationNote1 | Config 1, 2, 3 |  | N/A |  |
|  | Config 4, 5, 6 |  | 6 |  |
| TDD uplink-downlink configurationNote1 | Config 1, 2, 3 |  | N/A |  |
|  | Config 4, 5, 6 |  | 1 |  |
| BWchannel |  | MHz | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 | Config 1, 2, 3 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  | Config 4, 5, 6 |  | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 | Config 1, 2, 3 |  | 5 MHz: OP.19 FDD 10 MHz: OP.6 FDD 20 MHz: OP.14 FDD |  |
|  | Config 4, 5, 6 |  | 5 MHz: OP.10 TDD 10 MHz: OP.2 TDD 20 MHz: OP.8 TDD |  |
| PBCH_RA |  | dB | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | Bands FDD_A Note 9, TDD_A | dBm/15kHz | -91.65 | -117 |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  | -116.5 |
|  | Bands FDD_C, TDD_C |  |  | -116 |
|  | Bands FDD_D |  |  | -115.5 |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  | -115 |
|  | Bands FDD_G Note 8 |  |  | -114 |
|  | Bands FDD_H |  |  | -113.5 |
| Ês/Noc |  | dB | 10 | -4 |
| Ês/IotNote5 |  | dB | 10 | -4 |
| RSRPNote5 | Bands FDD_A Note 9, TDD_A | dBm/15kHz | -81.65 | -121 |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  | -120.5 |
|  | Bands FDD_C, TDD_C |  |  | -120 |
|  | Bands FDD_D |  |  | -119.5 |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  | -119 |
|  | Bands FDD_G Note 8 |  |  | -118 |
|  | Bands FDD_H |  |  | -117.5 |
| SCH_RPNote5 | Bands FDD_A Note 9, TDD_A | dBm/15kHz | -81.65 | -121 |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  | -120.5 |
|  | Bands FDD_C, TDD_C |  |  | -120 |
|  | Bands FDD_D |  |  | -119.5 |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  | -119 |
|  | Bands FDD_G Note 8 |  |  | -118 |
|  | Bands FDD_H |  |  | -117.5 |
| IoNote5 | Bands FDD_A Note 9, TDD_A | dBm/Ch BW | -53.45 + 10log(NRB,c/50) | -87.76 + 10log(NRB,c/50) |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  | -87.26 + 10log(NRB,c/50) |
|  | Bands FDD_C, TDD_C |  |  | -86.76 + 10log(NRB,c/50) |
|  | Bands FDD_D |  |  | -86.26 + 10log(NRB,c/50) |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  | -85.76 + 10log(NRB,c/50) |
|  | Bands FDD_G Note 8 |  |  | -84.76 + 10log(NRB,c/50) |
|  | Bands FDD_H |  |  | -84.26 + 10log(NRB,c/50) |
| Propagation Condition |  |  | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: E-UTRA operating band groups are as defined in clause 3.5 of TS 36.133 [15]. Note 7: For Band 26, the tests shall be performed with the carrier frequency of assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Except Band 29. Note 9: Except Band 32, Band 75 and Band 76. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |

##### A.6.7.5.1.3	Test Requirements

The SA inter-RAT E-UTRAN RSRP measurement accuracy for cell 2 shall fulfil absolute requirement in clause 10.2.2.

### A.6.7.6	E-UTRAN RSRQ

#### A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell

##### A.6.7.6.1.1	Test Purpose and Environment

The purpose of this test is to verify that the E-UTRAN RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.2.3 for SA inter-RAT E-UTRAN RSRQ measurements.

##### A.6.7.6.1.2	Test parameters

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an E-UTRAN inter-RAT neighbour cell. Supported test configurations are shown in table A.6.7.6.1.2-1. The measurement accuracy of SA inter-RAT E-UTRAN RSRQ are tested by using the parameters in A.6.7.6.1.2-2 and A.6.7.6.1.2-3.

Table A.6.7.6.1.2-1: Inter-RAT E-UTRAN RSRQ supported test configurations with FR1 serving cell

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, E-UTRAN FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, E-UTRAN FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, E-UTRAN FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, E-UTRAN TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, E-UTRAN TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, E-UTRAN TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.6.1.2-2: NR Cell specific test parameters for SA Inter-RAT E-UTRAN RSRQ test parameters

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| NR RF channel number |  |  | 1 |
| Duplex mode | Config 1, 4 |  | FDD |
|  | Config 2, 3, 5, 6 |  | TDD |
| TDD Configuration | Config 1, 4 |  | N/A |
|  | Config 2, 5 |  | TDDConf.1.1 |
|  | Config 3, 6 |  | TDDConf.2.1 |
| BWchannel | Config 1, 4 | MHz | 10: NRB,c = 52 (FDD) |
|  | Config 2, 5 |  | 10: NRB,c = 52 (TDD) |
|  | Config 3, 6 |  | 40: NRB,c = 106 (TDD) |
| Gap pattern Id |  |  | 0 |
| PDSCH reference measurement channel | Config 1, 4 |  | SR.1.1 FDD |
|  | Config 2, 5 |  | SR.1.1 TDD |
|  | Config 3, 6 |  | SR.2.1 TDD |
| RMSI CORSET reference channel | Config 1, 4 |  | CR.1.1 FDD |
|  | Config 2, 5 |  | CR.1.1 TDD |
|  | Config 3, 6 |  | CR.2.1 TDD |
| Dedicated CORSET reference channel | Config 1, 4 |  | CCR.1.1 FDD |
|  | Config 2, 5 |  | CCR.1.1 TDD |
|  | Config 3, 6 |  | CCR.2.1 TDD |
| CSI-RS for tracking | Config 1, 4 |  | TRS.1.1 FDD |
|  | Config 2, 5 |  | TRS.1.1 TDD |
|  | Config 3, 6 |  | TRS.1.2 TDD |
| BWP configurations | Initial DL BWP |  | DLBWP.0.1 |
|  | Dedicated DL BWP |  | DLBWP.1.1 |
|  | Initial UL BWP |  | ULBWP.0.1 |
|  | Dedicated UL BWP |  | ULBWP.1.1 |
| OCNG patternNote1 |  |  | OP.1 |
| SMTC configuration |  |  | SMTC.1 |
| SSB configuration | Config 1, 2, 4, 5 |  | SSB.1 FR1 |
|  | Config 3, 6 |  | SSB.2 FR1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |
| NocNote2 |  | dBm/15 kHz | -104 |
| NocNote2 | Config 1, 2, 4, 5 | dBm/SCS | -104 |
|  | Config 3, 6 |  | -101 |
| Ês/Noc |  | dB | dB |
| Ês/IotNote3 |  | dB | dB |
| SS-RSRQNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| SSB_RPNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| IoNote3 | Config 1, 2, 4, 5 | dBm/9.36 MHz | -58.96 |
|  | Config 3, 6 | dBm/38.16 MHz | -52.87 |
| Propagation condition |  |  | AWGN |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRQ, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table A.6.7.6.1.2-3: E-UTRAN Cell specific test parameters for SA Inter-RAT E-UTRAN RSRQ test parameters

| Parameter |  | Unit | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 |
| E-UTRA RF channel number |  |  | 1 |  |  |
| Duplex mode | Config 1, 2, 3 |  | FDD |  |  |
|  | Config 4, 5, 6 |  | TDD |  |  |
| TDD special subframe configurationNote1 | Config 1, 2, 3 |  | N/A |  |  |
|  | Config 4, 5, 6 |  | 6 |  |  |
| TDD uplink-downlink configurationNote1 | Config 1, 2, 3 |  | N/A |  |  |
|  | Config 4, 5, 6 |  | 1 |  |  |
| BWchannel |  | MHz | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 | Config 1, 2, 3 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |  |
|  | Config 4, 5, 6 |  | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |  |
| OCNG PatternsNote2 | Config 1, 2, 3 |  | 5 MHz: OP.19 FDD 10 MHz: OP.6 FDD 20 MHz: OP.14 FDD |  |  |
|  | Config 4, 5, 6 |  | 5 MHz: OP.10 TDD 10 MHz: OP.2 TDD 20 MHz: OP.8 TDD |  |  |
| PBCH_RA |  | dB | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |  |
| NocNote4 | Bands FDD_A Note 9, TDD_A | dBm/15kHz | -83 | -104.70 | -119.5 |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  | -119 |
|  | Bands FDD_C, TDD_C |  |  |  | -118.5 |
|  | Bands FDD_D |  |  |  | -118 |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  |  | -117.5 |
|  | Bands FDD_G Note 8 |  |  |  | -116.5 |
|  | Bands FDD_H |  |  |  | -116 |
| Ês/Noc |  | dB | -1.75 | -4.0 | -4.0 |
| Ês/IotNote5 |  | dB | -1.75 | -4.0 | -4.0 |
| RSRPNote5 | Bands FDD_A Note 9, TDD_A | dBm/15kHz | -84.75 | -108.70 | -123.5 |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  | -123 |
|  | Bands FDD_C, TDD_C |  |  |  | -122.5 |
|  | Bands FDD_D |  |  |  | -122 |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  |  | -121.5 |
|  | Bands FDD_G Note 8 |  |  |  | -120.5 |
|  | Bands FDD_H |  |  |  | -120 |
| RSRQNote5 | Bands FDD_A Note 9, TDD_A | dB | -14.76 | -16.25 | -16.25 |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |  |
|  | Bands FDD_C, TDD_C |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  |  |  |
|  | Bands FDD_G Note 8 |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |
| IoNote5 | Bands FDD_A Note 9, TDD_A | dBm/Ch BW | -53 + 10log(NRB,c /50) | -75.46 + 10log(NRB,c /50) | -90.26 + 10log(NRB,c /50) |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  | -89.76 + 10log(NRB,c /50) |
|  | Bands FDD_C, TDD_C |  |  |  | -89.26 + 10log(NRB,c /50) |
|  | Bands FDD_D |  |  |  | -88.76 + 10log(NRB,c /50) |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  |  | -88.26 + 10log(NRB,c /50) |
|  | Bands FDD_G Note 8 |  |  |  | -87.26 + 10log(NRB,c /50) |
|  | Bands FDD_H |  |  |  | -86.76 + 10log(NRB,c /50) |
| Propagation Condition |  |  | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, RSRQ and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: E-UTRA operating band groups are as defined in clause 3.5 of TS 36.133 [15]. Note 7: For Band 26, the tests shall be performed with the carrier frequency of assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Except Band 29. Note 9: Except Band 32, Band 75 and Band 76. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

##### A.6.7.6.1.3	Test Requirements

The SA inter-RAT E-UTRAN RSRQ measurement accuracy for cell 2 shall fulfil absolute requirement in clause 10.2.3.

### A.6.7.7	E-UTRAN RS-SINR

#### A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell

##### A.6.7.7.1.1	Test Purpose and Environment

The purpose of this test is to verify that the E-UTRAN RS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.2.4 for SA inter-RAT E-UTRAN RS-SINR measurements.

##### A.6.7.7.1.2	Test parameters

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an E-UTRAN inter-RAT neighbour cell. Supported test configurations are shown in table A.6.7.7.1.2-1. The measurement accuracy of SA inter-RAT E-UTRAN RS-SINR are tested by using the parameters in A.6.7.7.1.2-2 and A.6.7.7.1.2-3.

Table A.6.7.7.1.2-1: Inter-RAT E-UTRAN RS-SINR supported test configurations with FR1 serving cell

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, E-UTRAN FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, E-UTRAN FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, E-UTRAN FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, E-UTRAN TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, E-UTRAN TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, E-UTRAN TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.7.1.2-2: NR Cell specific test parameters for SA Inter-RAT E-UTRAN RS-SINR test parameters

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| NR RF channel number |  |  | 1 |
| Duplex mode | Config 1, 4 |  | FDD |
|  | Config 2, 3, 5, 6 |  | TDD |
| TDD Configuration | Config 1, 4 |  | N/A |
|  | Config 2, 5 |  | TDDConf.1.1 |
|  | Config 3, 6 |  | TDDConf.2.1 |
| BWchannel | Config 1, 4 | MHz | 10: NRB,c = 52 (FDD) |
|  | Config 2, 5 |  | 10: NRB,c = 52 (TDD) |
|  | Config 3, 6 |  | 40: NRB,c = 106 (TDD) |
| Gap pattern Id |  |  | 0 |
| PDSCH reference measurement channel | Config 1, 4 |  | SR.1.1 FDD |
|  | Config 2, 5 |  | SR.1.1 TDD |
|  | Config 3, 6 |  | SR.2.1 TDD |
| RMSI CORSET reference channel | Config 1, 4 |  | CR.1.1 FDD |
|  | Config 2, 5 |  | CR.1.1 TDD |
|  | Config 3, 6 |  | CR.2.1 TDD |
| Dedicated CORSET reference channel | Config 1, 4 |  | CCR.1.1 FDD |
|  | Config 2, 5 |  | CCR.1.1 TDD |
|  | Config 3, 6 |  | CCR.2.1 TDD |
| CSI-RS for tracking | Config 1, 4 |  | TRS.1.1 FDD |
|  | Config 2, 5 |  | TRS.1.1 TDD |
|  | Config 3, 6 |  | TRS.1.2 TDD |
| BWP configurations | Initial DL BWP |  | DLBWP.0.1 |
|  | Dedicated DL BWP |  | DLBWP.1.1 |
|  | Initial UL BWP |  | ULBWP.0.1 |
|  | Dedicated UL BWP |  | ULBWP.1.1 |
| OCNG patternNote1 |  |  | OP.1 |
| SMTC configuration |  |  | SMTC.1 |
| SSB configuration | Config 1, 2, 4, 5 |  | SSB.1 FR1 |
|  | Config 3, 6 |  | SSB.2 FR1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |
| NocNote2 |  | dBm/15 kHz | -104 |
| NocNote2 | Config 1, 2, 4, 5 | dBm/SCS | -104 |
|  | Config 3, 6 |  | -101 |
| Ês/Noc |  | dB | 17 |
| Ês/IotNote3 |  | dB | 17 |
| SS-RS-SINRNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| SSB_RPNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| IoNote3 | Config 1, 2, 4, 5 | dBm/9.36 MHz | -58.96 |
|  | Config 3, 6 | dBm/38.16 MHz | -52.87 |
| Propagation condition |  |  | AWGN |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RS-SINR, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table A.6.7.7.1.2-3: E-UTRAN Cell specific test parameters for SA Inter-RAT E-UTRAN RS-SINR test parameters

| Parameter |  | Unit | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 |
| E-UTRA RF channel number |  |  | 1 |  |  |
| Duplex mode | Config 1, 2, 3 |  | FDD |  |  |
|  | Config 4, 5, 6 |  | TDD |  |  |
| TDD special subframe configurationNote1 | Config 1, 2, 3 |  | N/A |  |  |
|  | Config 4, 5, 6 |  | 6 |  |  |
| TDD uplink-downlink configurationNote1 | Config 1, 2, 3 |  | N/A |  |  |
|  | Config 4, 5, 6 |  | 1 |  |  |
| BWchannel |  | MHz | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 | Config 1, 2, 3 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |  |
|  | Config 4, 5, 6 |  | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |  |
| OCNG PatternsNote2 | Config 1, 2, 3 |  | 5 MHz: OP.19 FDD 10 MHz: OP.6 FDD 20 MHz: OP.14 FDD |  |  |
|  | Config 4, 5, 6 |  | 5 MHz: OP.10 TDD 10 MHz: OP.2 TDD 20 MHz: OP.8 TDD |  |  |
| PBCH_RA |  | dB | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |  |
| NocNote4 | Bands FDD_A Note 9, TDD_A | dBm/15kHz | -88 | -108.50 | -119.5 |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  | -119 |
|  | Bands FDD_C, TDD_C |  |  |  | -118.5 |
|  | Bands FDD_D |  |  |  | -118 |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  |  | -117.5 |
|  | Bands FDD_G Note 8 |  |  |  | -116.5 |
|  | Bands FDD_H |  |  |  | -116 |
| CRS Ês/Noc1 |  | dB | -1.75 | 20.0 | -4.0 |
| CRS Ês/IotNote5 |  | dB | -1.75 | 20.0 | -4.0 |
| RSRPNote5 | Bands FDD_A Note 9, TDD_A | dBm/15kHz | -89.75 | -88.50 | -123.5 |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  | -123 |
|  | Bands FDD_C, TDD_C |  |  |  | -122.5 |
|  | Bands FDD_D |  |  |  | -122 |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  |  | -121.5 |
|  | Bands FDD_G Note 8 |  |  |  | -120.5 |
|  | Bands FDD_H |  |  |  | -120 |
| RS-SINRNote5 | Bands FDD_A Note 9, TDD_A | dB | -1.75 | 20 | -4.0 |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |  |
|  | Bands FDD_C, TDD_C |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  |  |  |
|  | Bands FDD_G Note 8 |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |
| IoNote5 | Bands FDD_A Note 9, TDD_A | dBm/Ch BW | -53.79 + 10log(NRB,c /50) | -60.56 + 10log(NRB,c /50) | -93.48 + 10log(NRB,c /50) |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  | -92.98 + 10log(NRB,c /50) |
|  | Bands FDD_C, TDD_C |  |  |  | -92.48 + 10log(NRB,c /50) |
|  | Bands FDD_D |  |  |  | -91.98 + 10log(NRB,c /50) |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  |  | -91.48 + 10log(NRB,c /50) |
|  | Bands FDD_G Note 8 |  |  |  | -90.48 + 10log(NRB,c /50) |
|  | Bands FDD_H |  |  |  | -89.98 + 10log(NRB,c /50) |
| Propagation Condition |  |  | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 4a: Void. Note 5: CRS Ês/Iot, RSRP, RS-SINR and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: E-UTRA operating band groups are as defined in clause 3.5 of TS 36.133 [15]. Note 7: For Band 26, the tests shall be performed with the carrier frequency of assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Except Band 29. Note 9: Except Band 32, Band 75 and Band 76. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

##### A.6.7.7.1.3	Test Requirements

The SA inter-RAT E-UTRAN RS-SINR measurement accuracy for cell 2 shall fulfil absolute requirement in clause 10.2.4.

### A.6.7.8	CLI measurements

#### A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell

##### A.6.7.8.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SRS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Clauses 10.1.22.1.1 with the testing configurations for NR cells in Table A.6.7.8.1.1-1.

Table A.6.7.8.1.1-1: Applicable NR configurations for FR1 SRS-RSRP accuracy test

| Config | Description |
| --- | --- |
| 1 | 15kHz SRS SCS, 10 MHz bandwidth, TDD duplex mode |
| 2 | 30kHz SRS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.6.7.8.1.2	Test parameters

In this set of test cases there is one cell in the test, FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.6.7.8.1.2-1 below. The test parameter for the (virtual) neighbor cell UE transmitting SRS are given in Table A.6.7.8.1.2-2.

Before the test UE is configured to perform SRS-RSRP measurement. During the test, the test system transmits SRS resources for measurement in the DL slots according to the SRS configuration in Table A.6.7.8.1.2-3. There is no measurement gap configured in the test. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on SRS symbol to be transmitted and on 1 data symbol before SRS to be transmitted.

Table A.6.7.8.1.2-1: FR1 test parameters for SRS-RSRP accuracy for PCell

| Parameter |  | Config | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1~2 |  | freq1 | freq1 | freq1 |
| Duplex mode |  | 1~2 |  | TDD | TDD | TDD |
| TDD configuration |  | 1 |  | TDDConf.1.1 | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 2 |  | TDDConf.2.1 | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 2 |  | 40: NRB,c = 106 | 40: NRB,c = 106 | 40: NRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 TDD | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 2 |  | SR.2.1 TDD | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 TDD | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 2 |  | CR.2.1 TDD | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 TDD | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 2 |  | CCR.2.1 TDD | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  |  | 2 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |
| OCNG Patterns |  | 1~2 |  | OP.1 | OP.1 | OP.1 |
| TRS configuration |  | 1 |  | TRS.1.1 TDD | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 2 |  | TRS.1.2 TDD | TRS.1.2 TDD | TRS.1.2 TDD |
| Initial BWP Configuration |  | 1~2 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1~2 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~2 |  | SMTC.1 | SMTC.1 | SMTC.1 |
| Time offset between DL from serving cell and SRS from test system |  | 1~2 | s | 17.67 | 17.67 | 17.67 |
| EPRE ratio of PSS to SSS |  | 1~2 | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Note2 | NR_TDD_FR1_A NOTE 3 | 1 | dBm/15kHz | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A NOTE 5 | 2 |  | Not applicableNote 4 | -91 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
| Note2 | NR_TDD_FR1_A NOTE 3 | 1 | dBm/SRS SCS | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A NOTE 3 | 2 |  | Not applicableNote 4 | -88 | -111 |
|  | NR_TDD_FR1_C |  |  |  |  | -110 |
|  | NR_TDD_FR1_D |  |  |  |  | -109.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -109 |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification Note 4: Test 1 is not used when testing with 30kHz SSB SCS |  |  |  |  |  |  |

Table A.6.7.8.1.2-2: FR1 test parameters for SRS-RSRP accuracy for neighbour cell UE

| Parameter |  | Config | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Note2 | NR_TDD_FR1_A NOTE 5 | 1 | dBm/15kHz | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A NOTE 5 | 2 |  | Not applicableNote 6 | -91 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
| Note2 | NR_TDD_FR1_A NOTE 5 | 1 | dBm/SRS SCS | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A NOTE 5 | 2 |  | Not applicableNote 6 | -88 | -111 |
|  | NR_TDD_FR1_C |  |  |  |  | -110 |
|  | NR_TDD_FR1_D |  |  |  |  | -109.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -109 |
| on SRS |  | 1~2 | dB | 1 | 1 | 1 |
| SRS RSRP Note3 | NR_TDD_FR1_A NOTE 5 | 1 | dBm/SRS SCS | -105 | -87 | -113 |
|  | NR_TDD_FR1_C |  |  |  |  | -112 |
|  | NR_TDD_FR1_D |  |  |  |  | -111.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -111 |
|  | NR_TDD_FR1_A NOTE 5 | 2 |  | Not applicableNote 6 | -87 | -110 |
|  | NR_TDD_FR1_C |  |  |  |  | -109 |
|  | NR_TDD_FR1_D |  |  |  |  | -108.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -108 |
| Io Note3 | NR_TDD_FR1_A NOTE 5 | 1 | dBm/9.36 MHz | -74.51 | -56.51 | -82.51 |
|  | NR_TDD_FR1_C |  |  |  |  | -81.51 |
|  | NR_TDD_FR1_D |  |  |  |  | -81.01 |
|  | NR_TDD_FR1_E |  |  |  |  | -79.51 |
|  | NR_TDD_FR1_A NOTE 5 | 2 | dBm/38.16 MHz | Not applicableNote 6 | -53.42 | -76.42 |
|  | NR_TDD_FR1_C |  |  |  |  | -75.42 |
|  | NR_TDD_FR1_D |  |  |  |  | -74.92 |
|  | NR_TDD_FR1_E |  |  |  |  | -74.42 |
| on SRS |  | 1~2 | dB | 1 | 1 | 1 |
| Propagation condition |  | 1~2 |  | AWGN | AWGN | AWGN |
| Antenna configuration |  | 1~2 |  | 1x2 | 1x2 | 1x2 |
| SRS configuration |  | 1 |  | SRSConf.1 | SRSConf.1 | SRSConf.1 |
|  |  | 2 |  | SRSConf.2 | SRSConf.2 | SRSConf.2 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of the test. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification Note 6: Test 1 is not used when testing with 30kHz SSB SCS |  |  |  |  |  |  |

Table A.6.7.8.1.2-3: SRS configuration parameters for FR1 SRS-RSRP accuracy

|  | Field | SRSConf.1 | SRSConf.2 |
| --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 0 |
|  | srs-ResourceIdList | 0 | 0 |
|  | resourceType | Periodic | Periodic |
|  | Usage | Codebook | Codebook |
| SRS-Resource | SRS-ResourceId | 0 | 0 |
|  | nrofSRS-Ports | Port1 | Port1 |
|  | transmissionComb | n2 | n2 |
|  | combOffset-n2 | 0 | 0 |
|  | cyclicShift-n2 | 0 | 0 |
|  | resourceMapping startPosition | 0 | 0 |
|  | resourceMapping nrofSymbols | n1 | n1 |
|  | resourceMapping repetitionFactor | n1 | n1 |
|  | freqDomainPosition | 0 | 0 |
|  | freqDomainShift | 0 | 0 |
|  | freqHopping c-SRS | 12 | 12 |
|  | freqHopping b-SRS | 0 | 0 |
|  | freqHopping b-hop | 0 | 0 |
|  | groupOrSequenceHopping | Neither | Neither |
|  | resourceType | Periodic | Periodic |
|  | periodicityAndOffset-p | sl20, 9 | sl40, 19 |
|  | sequenceId | 0 | 0 |

#### A.6.7.8.1.3	Test Requirements

The SRS-RSRP measurement accuracy shall fulfil the requirements in clauses 10.1.22.1.1.

#### A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell

##### A.6.7.8.2.1	Test Purpose and Environment

The purpose of this test is to verify that the CLI-RSSI measurement accuracy is within the specified limits. This test will verify the requirements in Clauses 10.1.22.2.1 with the testing configurations for NR cells in Table A.6.7.8.2.1-1.

Table A.6.7.8.2.1-1: Applicable NR configurations for FR1 CLI-RSSI accuracy test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SRS SCS, 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 30kHz SRS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.6.7.8.2.2	Test parameters

In this set of test cases there is one cell in the test, the FR1 PSCell (Cell 1). The test parameters for the Cell 1 are given in Table A.6.7.8.2.2-1 below.

Before the test UE is configured to perform CLI-RSSI measurement. There is no measurement gap configured in the test. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on symbols for CLI-RSSI resource and on 1 data symbol before. The CLI-RSSI measurement resource configuration is in Table A.6.7.8.2.2-2.

Table A.6.7.8.2.2-1: FR1 test parameters for CLI-RSSI accuracy

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~2 |  | freq1 |
| Duplex mode | 1~2 |  | TDD |
| TDD configuration | 1 |  | TDDConf.1.1 |
|  | 2 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 TDD |
|  | 2 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 TDD |
|  | 2 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 TDD |
|  | 2 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.1 FR1 |
|  | 2 |  | SSB.2 FR1 |
| OCNG Patterns Note6 | 1~2 |  | OP.1 |
| TRS configuration | 1 |  | TRS.1.1 TDD |
|  | 2 |  | TRS.1.2 TDD |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~2 |  | SMTC.1 |
| Time offset between DL from serving cell and OCNG from test system | 1~2 | s | 17.67 |
| EPRE ratio of PSS to SSS | 1~2 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| on CLI-RSSI measurement resource Note2 | 1 | dBm/15kHz | -106 |
|  | 2 |  | -106 |
| on CLI-RSSI measurement resource Note2 | 1 | dBm/ BWP SCS | -106 |
|  | 2 |  | -103 |
| on CLI-RSSI measurement resource | 1~2 | dB | -Infinity |
| RSRP on CLI-RSSI measurement resource Note3 | 1~2 | dBm/ BWP SCS | -Infinity |
| Io on CLI-RSSI measurement resource Note3 | 1 | dBm/9.36 MHz | -78.05 |
|  | 2 | dBm/38.16 MHz | -71.96 |
| Io on CLI-RSSI measurement resource Note3 | 1 | dBm/1.08 MHz | -87.43 |
|  | 2 |  | -87.44 |
| on CLI-RSSI measurement resource | 1~2 | dB | -Infinity |
| Propagation condition | 1~2 |  | AWGN |
| Antenna configuration | 1~2 |  | 1x2 |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification Note 6: OCNG is not transmitted in the CLI-RSSI measurement resources. |  |  |  |

Table A.6.7.8.2.2-2: CLI-RSSI measurement resource configuration for FR1 CLI-RSSI accuracy

|  | Field | Config | SRSConf.1 |
| --- | --- | --- | --- |
| CLI-RSSI measurement resource | rssi-ResourceId | 1~2 | 0 |
|  | rssi-SCS | 1 | 15kHz |
|  |  | 2 | 30kHz |
|  | startPRB | 1~2 | 0 |
|  | nrofPRBs | 1 | 52 |
|  |  | 2 | 106 |
|  | startPosition | 1~2 | 3 |
|  | nrofSymbols | 1~2 | 11 |
|  | rssi-PeriodicityAndOffset | 1 | sl20, 9 |
|  |  | 2 | sl40, 19 |

##### A.6.7.8.2.3	Test Requirements

The CLI-RSSI measurement accuracy shall fulfil the requirements in clauses 10.1.22.2.1.

### A.6.7.9	L1-SINR measurement for beam reporting

#### A.6.7.9.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off

##### A.6.7.9.1.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clause 9.8.4.1 and clause 10.1.27.1 for L1-SINR measurements based on CSI-RS with the testing configurations for NR cells in Table A.6.7.9.1.1-1.

Table A.6.7.9.1.1-1: Applicable NR configurations for FR1 L1-SINR test with CSI-RS based CMR and no dedicated IMR configured

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.6.7.9.1.2	Test parameters

In this set of test cases there are one cell in the test, PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.6.7.9.1.2-1 below. The absolute and relative accuracy of L1-SINR measurements are tested by using the parameters in Table A.6.7.9.1.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured one CSI-RS resource set with two CSI-RS resources. UE is configured to perform RLM and BFD based on SSB 0 and 1. CSI-RS is not transmitted in the same OFDM symbols as SSB.

Table A.6.7.9.1.2-1: FR1 CSI-RS based L1-SINR test parameters

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1~3 |  | freq1 | freq1 |
| Duplex mode |  | 1 |  | FDD | FDD |
|  |  | 2 |  | TDD | TDD |
|  |  | 3 |  | TDD | TDD |
| TDD Configuration |  | 1 |  | N/A | N/A |
|  |  | 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 2 |  | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 3 |  | 40: NRB,c = 106 | 40: NRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  |  | 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 3 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  |  | 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 3 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 3 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.1 FR1 | SSB.1 FR1 |
|  |  | 2 |  | SSB.1 FR1 | SSB.1 FR1 |
|  |  | 3 |  | SSB.2 FR1 | SSB.2 FR1 |
| OCNG Patterns |  | 1~3 |  | OP.1 | OP.1 |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  |  | 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 3 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~3 |  | SMTC.1 | SMTC.1 |
| CSI-RS |  | 1 |  | CSI-RS 1.2 FDD | CSI-RS 1.2 FDD |
|  |  | 2 |  | CSI-RS 1.2 TDD | CSI-RS 1.2 TDD |
|  |  | 3 |  | CSI-RS 2.2 TDD | CSI-RS 2.2 FDD |
| reportConfigType |  | 1~3 |  | periodic | periodic |
| reportQuantity-r16 |  | 1~3 |  | cri-SINR-r16 | cri-SINR-r16 |
| nrofReportedRS |  | 1~3 |  | 2 | 2 |
| L1-SINR reporting period |  | 1~3 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/15kHz | -94.65 | -117 |
|  | NR_FDD_FR1_B |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/CSI-RS SCS | -94.65 | -117 |
|  | NR_FDD_FR1_B |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -91.65 | -114 |
|  | NR_FDD_FR1_B |  |  |  | -113.5 |
|  | NR_TDD_FR1_C |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -112.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -112 |
|  | NR_FDD_FR1_F |  |  |  | -111.5 |
|  | NR_FDD_FR1_G |  |  |  | -111 |
|  | NR_FDD_FR1_H |  |  |  | -110.5 |
|  |  | 1~3 | dB | 10 | -3 |
| CSI-RS RSRP Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/CSI-RS SCS | -84.65 | -120 |
|  | NR_FDD_FR1_B |  |  |  | -119.5 |
|  | NR_TDD_FR1_C |  |  |  | -119 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -118.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -118 |
|  | NR_FDD_FR1_F |  |  |  | -117.5 |
|  | NR_FDD_FR1_G |  |  |  | -117 |
|  | NR_FDD_FR1_H |  |  |  | -116.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -81.65 | -117 |
|  | NR_FDD_FR1_B |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
| Io Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/9.36 MHz | -56.28 | -87.28 |
|  | NR_FDD_FR1_B |  |  |  | -86.78 |
|  | NR_TDD_FR1_C |  |  |  | -86.28 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -85.78 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -85.28 |
|  | NR_FDD_FR1_F |  |  |  | -84.78 |
|  | NR_FDD_FR1_G |  |  |  | -84.28 |
|  | NR_FDD_FR1_H |  |  |  | -83.78 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 | dBm/38.16 MHz | -50.19 | -81.19 |
|  | NR_FDD_FR1_B |  |  |  | -80.69 |
|  | NR_TDD_FR1_C |  |  |  | -80.19 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -79.69 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -79.19 |
|  | NR_FDD_FR1_F |  |  |  | -78.69 |
|  | NR_FDD_FR1_G |  |  |  | -78.19 |
|  | NR_FDD_FR1_H |  |  |  | -77.69 |
|  |  | 1~3 | dB | 10 | -3 |
| Propagation condition |  | 1~3 |  | AWGN | AWGN |
| Antenna configuration |  | 1~3 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |

##### A.6.7.9.1.3	Test Requirements

The L1-SINR measurement accuracy for CSI-RS#0 and CSI-RS#1 of Cell 1 shall fulfil the requirements in clause 10.1.27.1.

#### A.6.7.9.2	L1-SINR measurement with SSB based CMR and dedicated IMR

##### A.6.7.9.2.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clause 9.8.4.2 and clause 10.1.27.2 for L1-SINR measurements with SSB based CMR and dedicated CSI-RS based IMR, with the testing configurations for NR cells in Table A.6.7.9.2.1-1.

Table A.6.7.9.2.1-1: Applicable NR configurations for FR1 L1-SINR measurement test with SSB based CMR and CSI-RS based IMR

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.6.7.9.2.2	Test parameters

In this set of test cases there one cell in the test, PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.6.7.9.2.2-1 below. The absolute accuracy of L1-SINR measurements are tested by using the parameters in Table A.6.7.9.2.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured one SSB resource set with two SSB resources and one CSI-RS resource set with two CSI-RS resource. UE is configured to perform RLM and BFD measurement based on the SSB resources 0 and 1. UE is configured to perform L1-SINR measurement based on the SSBs as CMR and the CSI-RS resources as IMR.

Table A.6.7.9.2.2-1: FR1 SSB based L1-SINR test parameters

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1~3 |  | freq1 | freq1 |
| Duplex mode |  | 1 |  | FDD | FDD |
|  |  | 2 |  | TDD | TDD |
|  |  | 3 |  | TDD | TDD |
| TDD Configuration |  | 1 |  | N/A | N/A |
|  |  | 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 2 |  | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 3 |  | 40: NRB,c = 106 | 40: NRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  |  | 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 3 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  |  | 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 3 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 3 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.3 FR1 | SSB.3 FR1 |
|  |  | 2 |  | SSB.3 FR1 | SSB.3 FR1 |
|  |  | 3 |  | SSB.4 FR1 | SSB.4 FR1 |
| CSI-RS configuration |  | 1 |  | CSI-RS 1.1A FDD | CSI-RS 1.1A FDD |
|  |  | 2 |  | CSI-RS 1.1A TDD | CSI-RS 1.1A TDD |
|  |  | 3 |  | CSI-RS 2.1A TDD | CSI-RS 2.1A TDD |
| OCNG Patterns |  | 1~3 |  | OP.1 | OP.1 |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  |  | 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 3 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~3 |  | SMTC.1 | SMTC.1 |
| reportConfigType |  | 1~3 |  | periodic | periodic |
| reportQuantity-r16 |  | 1~3 |  | ssb-Index-SINR-r16 | ssb-Index-SINR-r16 |
| Number of reported RS |  | 1~3 |  | 2 | 2 |
| L1-SINR reporting period |  | 1~3 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/15kHz | -94.65 | -117 |
|  | NR_FDD_FR1_B |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/SSB SCS | -94.65 | -117 |
|  | NR_FDD_FR1_B |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -91.65 | -114 |
|  | NR_FDD_FR1_B |  |  |  | -113.5 |
|  | NR_TDD_FR1_C |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -112.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -112 |
|  | NR_FDD_FR1_F |  |  |  | -111.5 |
|  | NR_FDD_FR1_G |  |  |  | -111 |
|  | NR_FDD_FR1_H |  |  |  | -110.5 |
|  |  | 1~3 | dB | 10 | 0 |
|  |  | 1~6 | dB | 10 | 0 |
| SSB RSRP Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/SSB SCS | -84.65 | -117 |
|  | NR_FDD_FR1_B |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -81.65 | -114 |
|  | NR_FDD_FR1_B |  |  |  | -113.5 |
|  | NR_TDD_FR1_C |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -112.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -112 |
|  | NR_FDD_FR1_F |  |  |  | -111.5 |
|  | NR_FDD_FR1_G |  |  |  | -111 |
|  | NR_FDD_FR1_H |  |  |  | -110.5 |
| CSI-RS RSRP Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/CSI-RS SCS | -84.65 | -117 |
|  | NR_FDD_FR1_B |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -81.65 | -114 |
|  | NR_FDD_FR1_B |  |  |  | -113.5 |
|  | NR_TDD_FR1_C |  |  |  | -113 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -112.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -112 |
|  | NR_FDD_FR1_F |  |  |  | -111.5 |
|  | NR_FDD_FR1_G |  |  |  | -111 |
|  | NR_FDD_FR1_H |  |  |  | -110.5 |
| Io Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/9.36 MHz | -56.28 | -86.04 |
|  | NR_FDD_FR1_B |  |  |  | -85.54 |
|  | NR_TDD_FR1_C |  |  |  | -85.04 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -84.54 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -84.04 |
|  | NR_FDD_FR1_F |  |  |  | -83.54 |
|  | NR_FDD_FR1_G |  |  |  | -83.04 |
|  | NR_FDD_FR1_H |  |  |  | -82.54 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 | dBm/38.16 MHz | -50.19 | -79.94 |
|  | NR_FDD_FR1_B |  |  |  | -79.44 |
|  | NR_TDD_FR1_C |  |  |  | -78.94 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -78.44 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -77.94 |
|  | NR_FDD_FR1_F |  |  |  | -77.44 |
|  | NR_FDD_FR1_G |  |  |  | -76.94 |
|  | NR_FDD_FR1_H |  |  |  | -76.44 |
| Propagation condition |  | 1~3 |  | AWGN | AWGN |
| Antenna configuration |  | 1~3 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |

##### A.6.7.9.2.3	Test Requirements

The L1-SINR measurement accuracy for SSB#0+CSI-RS#0 and SSB#1+CSI-RS#1 of Cell 1 shall fulfil the requirements in clauses 10.1.27.2.

#### A.6.7.9.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR

##### A.6.7.9.3.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-SINR measurement accuracy is within the specified limits. This test will partly verify the requirements in Clauses 9.8.4.3 and clause 10.1.27.3 for L1-SINR measurements based on CSI-RS as CMR and CSI-IM as IMR with the testing configurations for NR cells in Table A.6.7.9.3.1-1.

Table A.6.7.9.3.1-1: Applicable NR configurations for FR1 L1-SINR measurement test with CSI-RS based CMR and CSI-IM based IMR

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.6.7.9.3.2	Test parameters

In this set of test cases there are one cell in the test, PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.6.7.9.3.2-1 below. The absolute and relative accuracy of L1-SINR measurements are tested by using the parameters in Table A.6.7.9.3.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured one CSI-RS resource set with two CSI-RS resources and one CSI-IM resource set with two CSI-IM resources. UE is configured to perform RLM and BFD based on SSB 0 and 1. CSI-RS is not transmitted in the same OFDM symbols as SSB. UE is configured to perform L1-SINR measurement based on the configured CSI-RS as CMR and CSI-IM as IMR.

Table A.6.7.9.3.2-1: FR2 L1-SINR measurement test with CSI-RS based CMR and CSI-IM based IMR

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1~3 |  | freq1 | freq1 |
| Duplex mode |  | 1 |  | FDD | FDD |
|  |  | 2 |  | TDD | TDD |
|  |  | 3 |  | TDD | TDD |
| TDD Configuration |  | 1 |  | N/A | N/A |
|  |  | 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 2 |  | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 3 |  | 40: NRB,c = 106 | 40: NRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  |  | 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 3 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  |  | 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 3 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 3 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.1 FR1 | SSB.1 FR1 |
|  |  | 2 |  | SSB.1 FR1 | SSB.1 FR1 |
|  |  | 3 |  | SSB.2 FR1 | SSB.2 FR1 |
| OCNG Patterns |  | 1~3 |  | OP.1 | OP.1 |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  |  | 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 3 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~3 |  | SMTC.1 | SMTC.1 |
| CSI-RS configuration as CMR |  | 1 |  | CSI-RS 1.2 FDD | CSI-RS 1.2 FDD |
|  |  | 2 |  | CSI-RS 1.2 TDD | CSI-RS 1.2 TDD |
|  |  | 3 |  | CSI-RS 2.2 TDD | CSI-RS 2.2 FDD |
| CSI-IM configuration as IMR |  | 1 |  | CSI-IM 1.3 FDD | CSI-IM 1.3 FDD |
|  |  | 2 |  | CSI-IM 1.3 TDD | CSI-IM 1.3 TDD |
|  |  | 3 |  | CSI-IM 2.3 TDD | CSI-IM 2.3 TDD |
| reportConfigType |  | 1~3 |  | periodic | periodic |
| reportQuantity-r16 |  | 1~3 |  | cri-SINR-r16 | cri-SINR-r16 |
| nrofReportedRS |  | 1~3 |  | 2 | 2 |
| L1-RSRP reporting period |  | 1~3 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/15kHz | -94.65 | -117 |
|  | NR_FDD_FR1_B |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/CSI-RS SCS | -94.65 | -117 |
|  | NR_FDD_FR1_B |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -91.65 | -114 |
|  | NR_FDD_FR1_B |  |  |  | -113.5 |
|  | NR_TDD_FR1_C |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -112.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -112 |
|  | NR_FDD_FR1_F |  |  |  | -111.5 |
|  | NR_FDD_FR1_G |  |  |  | -111 |
|  | NR_FDD_FR1_H |  |  |  | -110.5 |
|  |  | 1~3 | dB | 10 | -3 |
| CSI-RS RSRP Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/CSI-RS SCS | -84.65 | -120 |
|  | NR_FDD_FR1_B |  |  |  | -119.5 |
|  | NR_TDD_FR1_C |  |  |  | -119 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -118.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -118 |
|  | NR_FDD_FR1_F |  |  |  | -117.5 |
|  | NR_FDD_FR1_G |  |  |  | -117 |
|  | NR_FDD_FR1_H |  |  |  | -116.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -81.65 | -117 |
|  | NR_FDD_FR1_B |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
| Io Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/9.36 MHz | -56.28 | -87.28 |
|  | NR_FDD_FR1_B |  |  |  | -86.78 |
|  | NR_TDD_FR1_C |  |  |  | -86.28 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -85.78 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -85.28 |
|  | NR_FDD_FR1_F |  |  |  | -84.78 |
|  | NR_FDD_FR1_G |  |  |  | -84.28 |
|  | NR_FDD_FR1_H |  |  |  | -83.78 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 | dBm/38.16 MHz | -50.19 | -81.19 |
|  | NR_FDD_FR1_B |  |  |  | -80.69 |
|  | NR_TDD_FR1_C |  |  |  | -80.19 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -79.69 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -79.19 |
|  | NR_FDD_FR1_F |  |  |  | -78.69 |
|  | NR_FDD_FR1_G |  |  |  | -78.19 |
|  | NR_FDD_FR1_H |  |  |  | -77.69 |
|  |  | 1~3 | dB | 10 | -3 |
| Propagation condition |  | 1~3 |  | AWGN | AWGN |
| Antenna configuration |  | 1~3 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |

##### A.6.7.9.3.3	Test Requirements

The L1-SINR measurement accuracy for CSI-RS#0+CSI-IM#0 and CSI-RS#1+CSI-IM# of Cell 1 shall fulfil the requirements in clause 10.1.27.3.

### A.6.7.10	CSI-RSRP

#### A.6.7.10.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.9.10.1	Test Purpose and Environment

The purpose of this test is to verify that the CSI-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.2.3.1 and 10.1.2.3.2 for CSI-RS intra-frequency measurements.

##### A.6.7.9.10.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Supported test configurations are shown in table A.6.7.10.1.2-1. Both absolute and relative accuracy of CSI-RSRP intra-frequency measurements are tested by using the parameters in A.6.7.10.1.2-2. In all test cases, Cell 1 is the PCell, and Cell 2 is the target cell.

Table A.6.7.10.1.2-1: CSI-RSRP intra frequency supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table A.6.7.10.1.2-2: CSI-RSRP intra frequency test parameters

| Parameter |  |  |  |  |  | Unit | Test 1 |  |  | Test 2 |  |  |  |  |  | Test 3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 |  | Cell 2 |  |  |  | Cell 1 |  |  | Cell 2 |  |  |
| Cell ID |  |  |  |  |  |  | 489 | 0 |  | 489 |  | 0 |  |  |  | 489 |  | 0 |  |  |  |
| SSB ARFCN |  |  |  |  |  |  | freq1 |  |  | freq1 |  |  |  |  |  | freq1 |  |  |  |  |  |
| Duplex mode |  |  |  | Config 1 |  |  | FDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 2,3 |  |  | TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TDD configuration |  |  |  | Config 1 |  |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 2 |  |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BWchannel |  |  |  | Config 1 |  | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 2 |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BWP BW |  |  |  | Config 1 |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 2 |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TRS configuration |  |  |  |  | Config 1 |  | TRS.1.1 FDD |  | NA |  | TRS.1.1 FDD |  |  | NA | TRS.1.1 FDD |  |  |  |  |  | NA |
|  |  |  |  |  | Config 2 |  | TRS.1.1 TDD |  | NA |  | TRS.1.1 TDD |  |  | NA | TRS.1.1 TDD |  |  |  |  |  | NA |
|  |  |  |  |  | Config 3 |  | TRS.1.2 TDD |  | NA |  | TRS.1.2 TDD |  |  | NA | TRS.1.2 TDD |  |  |  |  |  | NA |
| DRX Cycle |  |  |  |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  |  | Config 1 |  |  | SR.1.1 FDD |  | - | SR.1.1 FDD |  | - |  |  |  | SR.1.1 FDD |  |  |  |  | - |
|  |  |  |  | Config 2 |  |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |  |  |  | SR.1.1 TDD |  |  |  |  |  |
|  |  |  |  | Config 3 |  |  | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |  |  |  | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  |  | Config 1 |  |  | CR.1.1 FDD |  | - | CR.1.1 FDD |  | - |  |  |  | CR.1.1 FDD |  |  |  |  | - |
|  |  |  |  | Config 2 |  |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |  |  |  | CR.1.1 TDD |  |  |  |  |  |
|  |  |  |  | Config 3 |  |  | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |  |  |  | CR.2.1 TDD |  |  |  |  |  |
| Control channel RMC |  |  |  | Config 1 |  |  | CCR.1.1 FDD |  | - | CCR.1.1 FDD |  | - |  |  |  | CCR.1.1 FDD |  |  |  |  | - |
|  |  |  |  | Config 2 |  |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |  |  |  | CCR.1.1 TDD |  |  |  |  |  |
|  |  |  |  | Config 3 |  |  | CCR2.1 TDD |  |  | CCR2.1 TDD |  |  |  |  |  | CCR2.1 TDD |  |  |  |  |  |
| SSB configuration |  | Config 1 |  |  |  |  | SSB.1 FR1 |  | SSB.1 FR1 | SSB.1 FR1 |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |
|  |  | Config 2 |  |  |  |  | SSB.1 FR1 |  | SSB.1 FR1 | SSB.1 FR1 |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |
|  |  | Config 3 |  |  |  |  | SSB.2 FR1 |  | SSB.2 FR1 | SSB.2 FR1 |  | SSB.2 FR1 |  |  |  | SSB.2 FR1 |  |  |  | SSB.2 FR1 |  |
| Time offset with Cell 1 |  | Config 1,2 |  |  |  | s | - |  | 4.7 | - |  | 4.7 |  |  |  | - |  |  |  | 4.7 |  |
|  |  | Config 3 |  |  |  | s | - |  | 2.35 | - |  | 2.35 |  |  |  | - |  |  |  | 2.35 |  |
| SMTC configuration |  | Config 1 |  |  |  |  | SMTC.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  |  |  |  | SMTC.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CSI-RS configuration for RRM |  | Config 1 |  |  |  |  | CSI-RS.RRM.FR1.1 FDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  |  |  |  | CSI-RS.RRM.FR1.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  |  |  | CSI-RS.RRM.FR1.2 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  |  | Config 1,2 |  | kHz | 15 kHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  |  | 30kHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  | dB | 0 |  | 0 | 0 |  |  | 0 |  |  |  | 0 |  |  |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/15KhZ | -106 |  |  | -88 |  |  |  |  |  |  | -114 |  |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  | -113.5 |  |  |  |  |
|  |  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  | -113 |  |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  | -112.5 |  |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  | -112 |  |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  | -111.5 |  |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  | -111 |  |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  | -110.5 |  |  |  |  |
|  | Config 3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  |  | Not applicableNote 5 |  |  | -94 |  |  |  |  |  |  | -114 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  | -113.5 |  |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  | -113 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  | -112.5 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  | -112 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  | -111.5 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  | -111 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  | -110.5 |  |  |  |  |
| Note2 | Config 1,2 |  |  |  |  | dBm/SCS | -106 |  |  | -88 |  |  |  |  |  |  | Same as Noc/15kHz |  |  |  |  |
|  | Config 3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  |  | Not applicableNote 5 |  |  | -91 |  |  |  |  |  |  | -111 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  | -110.5 |  |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  | -110 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  | -109.5 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  | -109 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  | -108.5 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  | -108 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  | -107.5 |  |  |  |  |
|  |  |  |  |  |  | dB | 2.46 |  | -5.97 | 2.46 |  |  | -5.97 |  |  |  | -0.01 |  |  |  | -4.76 |
|  |  |  |  |  |  | dB | 6 |  | 1 | 6 |  |  | 1 |  |  |  | 3 |  |  |  | 0 |
| CSI-RSRPNote3 | Config 1,2 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  |  | dBm/SCS | -100 |  | -105 | -82 |  |  | -87 |  |  |  | -111.00 |  |  |  | -114.00 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  |  | -110.50 |  |  |  | -113.50 |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  |  | -110.00 |  |  |  | -113.00 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  |  | -109.50 |  |  |  | -112.50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  |  | -109.00 |  |  |  | -112.00 |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  |  | -108.50 |  |  |  | -111.50 |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  |  | -108.00 |  |  |  | -111.00 |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  |  | -107.50 |  |  |  | -110.50 |
|  | Config 3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  |  | Not applicableNote 5 |  | Not applicableNote 5 | -85 |  |  | -90 |  |  |  | -108.00 |  |  |  | -111.00 |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  | -107.50 |  |  |  | -110.50 |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  | -107.00 |  |  |  | -110.00 |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  | -106.50 |  |  |  | -109.50 |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  | -106.00 |  |  |  | -109.00 |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  | -105.50 |  |  |  | -108.50 |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  | -105.00 |  |  |  | -108.00 |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  | -104.50 |  |  |  | -107.50 |
| IoNote3 | Config 1,2 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | dBm/ 9.36MHz | -70.09 |  |  | -52.09 |  |  |  |  |  |  | -80.03 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  | -79.53 |  |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  | -79.03 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  | -78.53 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  | -78.03 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  | -77.53 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  | -77.03 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  | -76.53 |  |  |  |  |
|  | Config 3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | dBm/ 38.16MHz | Not applicableNote 5- |  |  | -51.99 |  |  |  |  |  |  | -73.94 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  | -73.44 |  |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  | -72.94 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  | -72.44 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  | -71.94 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  | -71.44 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  | -70.94 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  | -70.44 |  |  |  |  |
| Propagation condition |  |  |  |  |  | - | AWGN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Antenna configuration |  |  |  |  |  |  | 1x2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Subtest 1 is not used when testing with 30kHz SSB and CSI-RS SCS. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.7.10.1.3	Test Requirements

The CSI-RSRP measurement accuracy for cell 1 and cell 2 shall fulfil absolute requirement in clause 10.1.2.3.1 and relative requirement in clause 10.1.2.3.2.

#### A.6.7.10.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.9.10.1	Test Purpose and Environment

The purpose of this test is to verify that the CSI-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.4.3.1 and 10.1.4.3.2 for CSI-RS inter-frequency measurements with the testing configurations for NR cells in Table A.6.7.9.2.1-1.

Table A.6.7.10.2.1-1: Applicable NR configurations for FR1 inter-frequency CSI-RSRP accuracy test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.6.7.10.2.2	Test parameters

In this set of test cases there are two cells in the test, PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on a different frequency than the PCell. The test parameters for the Cell 1 and Cell 2 are given in Table A.6.7.10.2.2-1 below. Both absolute and relative accuracy of CSI-RSRP inter-frequency measurements are tested by using the parameters in Table A.6.7.10.2.2-1. The inter-frequency measurements are supported by a measurement gap.

Table A.6.7.10.2.2-1: CSI-RSRP inter-frequency test parameters

| Parameter |  | Config | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 |  | Cell 2 |
| SSB ARFCN |  | 1~3 |  | freq1 | freq2 |  | freq1 |  | freq2 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |  |
|  |  | 2 |  | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |  |
|  |  | 3 |  | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |  |
| Duplex mode |  | 1 |  | FDD |  |  | FDD |  |  |
|  |  | 2 |  | TDD |  |  | TDD |  |  |
|  |  | 3 |  | TDD |  |  | TDD |  |  |
| TDD configuration |  | 1 |  | N/A |  |  | N/A |  |  |
|  |  | 2 |  | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 |  | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | - |  | SR.1.1 FDD |  | - |
|  |  | 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 |  | SR.2.1 FDD |  |  | SR.2.1 FDD |  |  |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | - |  | CR.1.1 FDD |  | - |
|  |  | 2 |  | CR.1.1 TDD | - |  | CR.1.1 TDD |  | - |
|  |  | 3 |  | CR.2.1 FDD | - |  | CR.2.1 FDD |  | - |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD |  | - |
|  |  | 2 |  | CCR.1.1 TDD | - |  | CCR.1.1 TDD |  | - |
|  |  | 3 |  | CCR.2.1 TDD | - |  | CCR.2.1 TDD |  | - |
| SSB configuration |  | 1 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |
|  |  | 2 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |
|  |  | 3 |  | SSB.2 FR1 |  |  | SSB.2 FR1 |  |  |
| OCNG Patterns |  | 1~3 |  | OP.1 |  |  | OP.1 |  |  |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | - |  | TRS.1.1 FDD |  |  |
|  |  | 2 |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | 3 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |
| Time offset with Cell 1 |  | 1,2 | s | - |  | 4.7 | - | 4.7 |  |
|  |  | 3 | s | - |  | 2.35 | - | 2.35 |  |
| SMTC configuration |  | 1 |  | SMTC.2 |  |  | SMTC.2 |  |  |
|  |  | 2,3 |  | SMTC.1 |  |  | SMTC.1 |  |  |
| CSI-RS configuration for RRM |  | 1 |  | CSI-RS.RRM.FR1.1 FDD |  |  | CSI-RS.RRM.FR1.1 FDD |  |  |
|  |  | 2 |  | CSI-RS.RRM.FR1.1 TDD |  |  | CSI-RS.RRM.FR1.1 TDD |  |  |
|  |  | 3 |  | CSI-RS.RRM.FR1.2 TDD |  |  | CSI-RS.RRM.FR1.2 TDD |  |  |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 |  | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/15kHz | -94.65 |  |  | ( for Channel 2 +8dB) |  | -115 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -114.5 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -113.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -113 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -112.5 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -112 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -111.5 |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2 | dBm/SSB SCS | -94.65 |  |  | ( for Channel 2 +8dB) |  | -115 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -114.5 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -113.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -113 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -112.5 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -112 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -111.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3 |  | -91.65 |  |  | ( for Channel 2 +8dB) |  | -112.00 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -112.50 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -112.00 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -111.50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -111.00 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -110.50 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -110.00 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -110.50 |
|  |  | 1~3 | dB | 10 | 10 |  | 13 |  | -3 |
| CSI-RSRPNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2 | dBm/SCS | -84.65 |  |  | (RSRP for Cell 2 +25dB) |  | -118.00 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -117.50 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -117.00 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -116.50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -116.00 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -115.50 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -115.00 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -114.50 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3 |  | -81.65 |  |  | (RSRP for Cell 2 +25dB) |  | -115.00 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -114.50 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -114.00 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -113.50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -113.00 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -112.50 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -112.00 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -111.50 |
| IoNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2 | dBm/ 9.36MHz | -56.28 |  |  | Io for Channel 2 +19.75dB |  | -85.28 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -84.78 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -84.28 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -83.78 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -83.28 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -82.78 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -82.28 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -81.78 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3 | dBm/ 38.16MHz | -50.19 |  |  | Io for Channel 2 +19.75dB |  | -79.19 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -78.69 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -78.19 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -77.69 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -77.19 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -76.69 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -76.19 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -75.69 |
|  |  | 1~3 | dB | 10 | 10 |  | 13 |  | -3 |
| Propagation condition |  | 1~3 | - | AWGN |  |  | AWGN |  |  |
| Antenna configuration |  | 1~3 |  | 1x2 |  |  | 1x2 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |

##### A.6.7.10.2.3	Test Requirements

The CSI-RSRP measurement accuracy for Cell 1 and Cell 2 shall fulfil the absolute requirement in clause 10.1.4.3.1 and relative requirement in clause 10.1.4.3.2.

### A.6.7.11	CSI-RSRQ

#### A.6.7.11.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.11.1.1	Test Purpose and Environment

The purpose of this test is to verify that the CSI-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.7.2.

##### A.6.7.11.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. Supported test configuration are shown in Table A.6.7.11.1.2-1. The absolute accuracy of CSI-RSRQ intra-frequency measurement is tested by using the parameters in Table A.6.7.11.1.2-2. In all test cases, Cell 1 is the PCell and Cell 2 is the target cell.

Table A.6.7.11.1.2-1: Intra frequency CSI-RSRQ supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.11.1.2-2: CSI-RSRQ Intra frequency test parameters

| Parameter |  |  |  |  | Unit | Test 1 |  | Test 2 |  |  |  |  | Test 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Cell 1 | Cell 2 | Cell 1 |  |  | Cell 2 |  | Cell 1 |  | Cell 2 |
| Duplex mode |  |  |  | Config 1 |  | FDD |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |  |  |
| TDD configuration |  |  |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |
| BWchannel |  |  |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |
| Gap Pattern ID |  |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |
| BWP configuration |  |  |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |
| DRX Cycle |  |  |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  |  | Config 1 |  | SR.1.1 FDD | - | SR.1.1 FDD |  |  | - |  | SR.1.1 FDD |  | - |
|  |  |  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |  |  |  | SR.1.1 TDD |  |  |
|  |  |  |  | Config 3 |  | SR.2.1 TDD |  | SR.2.1 TDD |  |  |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  |  |  | Config 1 |  | CR.1.1 FDD | - | CR.1.1 FDD |  |  | - |  | CR.1.1 FDD |  |  |
|  |  |  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |  |  |  | CR.1.1 TDD |  |  |
|  |  |  |  | Config 3 |  | CR.2.1 TDD |  | CR.2.1 TDD |  |  |  |  | CR.2.1 TDD |  |  |
| Control Channel RMC |  |  |  | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD |  |  | - |  | CCR.1.1 FDD |  | - |
|  |  |  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |  |  | CCR.1.1 TDD |  |  |
|  |  |  |  | Config 3 |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |  |  |  | CCR.2.1 TDD |  |  |
| TRS Configuration |  |  |  | Config 1 |  | TRS.1.1 FDD | - | TRS.1.1 FDD |  |  | - |  | TRS.1.1 FDD |  | - |
|  |  |  |  | Config 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |  |  |  | TRS.1.1 TDD |  |  |
|  |  |  |  | Config 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |  |  |  | TRS.1.2 TDD |  |  |
| OCNG Patterns |  |  |  |  |  | OP. 1 |  |  |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 1 |  |  | s | - | 4.7 |  | - | 4.7 |  | - |  | 4.7 |  |
|  |  | Config 2,3 |  |  | s | - | 2.35 |  | - | 2.35 |  | - |  | 2.35 |  |
| CSI-RS configuration for RRM |  | Config 1 |  |  |  | CSI-RS.RRM.FR1.1 FDD |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  |  |  | CSI-RS.RRM.FR1.1 TDD |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  |  | CSI-RS.RRM.FR1.2 TDD |  |  |  |  |  |  |  |  |  |
| SMTC configuration |  | Config 1 |  |  |  | SMTC.2 |  |  |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  |  |  | SMTC.1 |  |  |  |  |  |  |  |  |  |
| SSB configuration |  |  |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  | 30kHz |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  | dB | 0 | 0 | 0 |  |  | 0 |  | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -85 |  | -101 |  |  |  |  | -114 |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -113.5 |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -113 |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -112.5 |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -112 |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  | -111.5 |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -111 |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -110.5 |  |  |
|  | Config 3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -91 |  | - |  |  |  |  | -114 |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -113.5 |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -113 |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -112.5 |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -112 |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  | -111.5 |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -111 |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -110.5 |  |  |
| Note2 | Config 1,2 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/SCS | -85 |  | -101 |  |  |  |  | -114 -113.5 -113 -112.5 -112 -111.5 -111 -110.5 |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |
|  | Config 3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -88 |  | - |  |  |  |  | -111 |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -110.5 |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -110 |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -109.5 |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -109 |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  | -108.5 |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -108 |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -107.5 |  |  |
|  |  |  |  |  | dB | -1.76 |  | -4.7 |  |  |  |  | -5..46 |  | -5.46 |
|  |  |  |  |  | dB | 3 | 3 | -2.9 |  |  | -2.9 |  | -4 |  | -4 |
| SS-RSRP/ CSI-RSRPNote3 | Config 1,2 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | dBm/SCS | -82 | -82 | -103.9 |  |  | -103.9 |  | -118 |  | -118 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  | -117.5 |  | -117.5 |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  | -117 |  | -117 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  | -116.5 |  | -116.5 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  | -116 |  | -116 |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  | -115.5 |  | -115.5 |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  | -115 |  | -115 |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  | -114.5 |  | -114.5 |
|  | Config 3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -85 | -85 | - |  |  | - |  | -115 |  | -115 |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -114.5 |  | -114.5 |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -114 |  | -114 |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -113.5 |  | -113.5 |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -113 |  | -113 |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  | -112.5 |  | -112.5 |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -112 |  | -112 |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -111.5 |  | -111.5 |
| SS-RSRQ/CSI-RSRQ Note3 |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dB | -14.77 | -14.77 | -16.76 |  |  | -16.76 |  | -17.34 |  | -17.34 |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |
| IoNote3 | Config 1,2 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 9.36MHz | -50 |  | -70 |  |  |  |  | -83.5 |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -83 |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -82.5 |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -82 |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -81.5 |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  | -81 |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -80.5 |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -80 |  |  |
|  | Config 3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 38.16MHz | -50 |  | - |  |  |  |  | -77.4 |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -76.9 |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -76.4 |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -75.9 |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -75.4 |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  | -74.9 |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -74.4 |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -73.9 |  |  |
| Propagation condition |  |  |  |  | - | AWGN | AWGN | AWGN |  |  | AWGN |  | AWGN |  | AWGN |
| Antenna configuration |  |  |  |  |  | 1x2 | 1x2 | 1x2 |  |  | 1x2 |  | 1x2 |  | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ/CSI-RSRQ, SS-RSRP/CSI-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ/CSI-RSRQ, SS-RSRP/CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.7.11.1.3	Test Requirements

The CSI-RSRQ measurement accuracy shall fulfil the requirements in clause 10.1.7.2.

#### A.6.7.11.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.11.2.1	Test Purpose and Environment

The purpose of this test is to verify that the CSI-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.9.2.1 and 10.1.9.2.2.

##### A.6.7.11.2.2	Test Parameters

In this test case the two cells (i.e., Cell 1 and Cell 2) are on different carrier frequencies and measurement gaps are provided. Supported test configurations are shown in Table A.6.7.11.2.2-1. Both absolute accuracy and relative accuracy requirements of CSI-RSRQ inter-frequency measurement are tested by using test parameters in Table A.6.7.11.2.2-2. In all test cases, Cell 1 is the PCell and Cell 2 is target cell.

Table A.6.7.11.2.2-1: CSI-RSRQ Inter frequency CSI-RSRQ supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.11.2.2-2: CSI-RSRQ Inter frequency test parameters

| Parameter |  |  |  | Unit | Test 1 |  |  |  | Test 2 |  |  |  |  | Test 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 1 |  | Cell 2 |  | Cell 1 |  |  | Cell 2 |  | Cell 1 |  | Cell 2 |  |
| Duplex mode |  | Config 1 |  |  | FDD |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  |  | TDD |  |  |  |  |  |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |  |  |  |
| BWchannel |  | Config 1 |  | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |  |  |
| Gap pattern ID |  | Config 1,2,3 |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |
| BWP BW |  | Config 1 |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |  |  |
| DRX Cycle |  |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  |  | SR.1.1 FDD | - |  |  | SR.1.1 FDD |  |  | - |  | SR.1.1 FDD |  | - |  |
|  |  | Config 2,5 |  |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |  |  | SR.1.1 TDD |  |  |  |
|  |  | Config 3,6 |  |  | SR.2.1 TDD |  |  |  | SR.2.1 TDD |  |  |  |  | SR.2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  |  | CR.1.1 FDD | - |  |  | R.1.1 FDD |  |  | - |  | CR.1.1 FDD |  |  |  |
|  |  | Config 2 |  |  | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |  |  | CR.1.1 TDD |  |  |  |
|  |  | Config 3 |  |  | CR2.1 TDD |  |  |  | CR2.1 TDD |  |  |  |  | CR2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  |  | CCR.1.1 FDD | - |  |  | CCR.1.1 FDD |  |  | - |  | CCR.1.1 FDD |  | - |  |
|  |  | Config 2 |  |  | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |  |  |  |  | CCR.1.1 TDD |  |  |  |
|  |  | Config 3 |  |  | CCR2.1 TDD |  |  |  | CCR2.1 TDD |  |  |  |  | CCR2.1 TDD |  |  |  |
| TRS Configuration |  | Config 1 |  |  | TRS.1.1 FDD | - |  |  | TRS.1.1 FDD |  |  | - |  | TRS.1.1 FDD |  | - |  |
|  |  | Config 2 |  |  | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  |  |  | TRS.1.1 TDD |  |  |  |
|  |  | Config 3 |  |  | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |  |  | TRS.1.2 TDD |  |  |  |
| OCNG Patterns |  |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 1,2 |  | s | - |  |  | 4.7 |  | - | 4.7 |  | - |  | 4.7 |  |  |
|  |  | Config 3 |  | s | - |  |  | 2.35 |  | - | 2.35 |  | - |  | 2.35 |  |  |
| CSI-RS configuration for RRM |  | Config 1 |  |  | CSI-RS.RRM.FR1.1 FDD |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  |  | CSI-RS.RRM.FR1.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | CSI-RS.RRM.FR1.2 TDD |  |  |  |  |  |  |  |  |  |  |  |  |
| SSB configuration |  | Config 1,2 |  |  | SSB.1 FR1 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | SSB.2 FR1 |  |  |  |  |  |  |  |  |  |  |  |  |
| SMTC configuration |  | Config 1,2 |  |  | SMTC.2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | SMTC.1 |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 |  | kHz | 15 kHz |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | 30 kHz |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 | 0 |  |  | 0 |  |  | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -80.18 |  |  |  | -106 |  |  |  |  | -116 |  |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  | -115.5 |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  | -115 |  |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  | -114.5 |  |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  | -114 |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  | -113.5 |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  | -113 |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  | -112.5 |  |  |  |
| Note2 | Config 3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -86.27 |  |  |  | -113 |  |  |  |  | -116 |  |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  | -115.5 |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  | -115 |  |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  | -114.5 |  |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  | -114 |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  | -113.5 |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  | -113 |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  | -112.5 |  |  |  |
| Note2 | Config 1,2 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -80.18 |  |  |  | -106 |  |  |  |  | -116 |  |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  | -115.5 |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  | -115 |  |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  | -114.5 |  |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  | -114 |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  | -113.5 |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  | -113 |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  | -112.5 |  |  |  |
|  | Config 3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -83.27 |  |  |  | -110 |  |  |  |  | -113 |  |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  | -112.5 |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  | -112 |  |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  | -111.5 |  |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  | -111 |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  | -110.5 |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  | -110 |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  | -109.5 |  |  |  |
|  |  |  |  | dB | -1.75 |  |  |  | -1.75 |  |  |  |  | 3 |  | -1.75 |  |
|  |  |  |  | dB | -1.75 |  |  |  | -1.75 |  |  |  |  | 3 |  | -1.75 |  |
| SS-RSRP/CSI-RSRPNote3 | Config 1,2 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/SCS | -81.93 | -81.93 |  |  | -107.75 |  |  | -107.75 |  | -113 |  |  | -117.75 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  | -112.5 |  |  | -117.25 |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  | -112 |  |  | -116.75 |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  | -111.5 |  |  | -116.25 |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  | -111 |  |  | -115.75 |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  | -110.5 |  |  | -115.2 |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  | -110 |  |  | -114.75 |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  | -109.5 |  |  | -114.25 |
|  | Config 3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -85.02 | -85.02 |  |  | -111.75 |  |  | -111.75 |  | -110 |  |  | -114.75 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  | -109.5 |  |  | -114.25 |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  | -109 |  |  | -113.75 |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  | -108.5 |  |  | -113.25 |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  | -108 |  |  | -112.75 |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  | -107.5 |  |  | -112.2 |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  | -107 |  |  | -111.75 |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  | -106.5 |  |  | -111.25 |
| SS-RSRQ/CSI-RSRQNote3 |  |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dB | -14.77 | -14.77 |  |  | -40.59 |  |  | -40.59 |  | 12.56T |  |  | 14.76T |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IoNote3 | Config 1,2 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/SCS | -50 |  |  |  | -75.83 |  |  |  |  | -83.28 |  |  | -85.83 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  | -82.78 |  |  | -85.33 |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  | -82.28 |  |  | -84.83 |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  | -81.78 |  |  | -84.33 |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  | -81.28 |  |  | -83.83 |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  | -80.78 |  |  | -83.33 |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  | -80.28 |  |  | -82.83 |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  | -79.78 |  |  | -82.33 |
|  | Config 3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -50 |  |  |  | -76.73 |  |  |  |  | -77.19 |  |  | -79.73 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  | -76.69 |  |  | -79.23 |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  | -76.19 |  |  | -78.73 |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  | -75.69 |  |  | -78.23 |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  | -75.19 |  |  | -77.73 |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  | -74.69 |  |  | -77.23 |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  | -74.19 |  |  | -76.73 |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  | -73.69 |  |  | -76.53 |
| Propagation condition |  |  |  | - | AWGN | AWGN |  |  | AWGN |  |  | AWGN |  | AWGN |  | AWGN |  |
| Antenna configuration |  |  |  |  | 1x2 | 1x2 |  |  | 1x2 |  |  | 1x2 |  | 1x2 |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ/CSI-RSRQ, SS-RSRP/CSI-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ/CSI-RSRQ, SS-RSRP/CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.7.11.2.3	Test Requirements

The CSI-RSRQ measurement accuracy shall fulfil the requirements in clause 10.1.9.2.1 and 10.1.9.2.2.

### A.6.7.12	CSI-SINR

#### A.6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.12.1.1	Test Purpose and Environment

The purpose of this test is to verify that the CSI-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.12.2.1.

##### A.6.7.12.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. Supported test configuration are shown in Table A.6.7.11.1.2-1. The absolute accuracy of CSI-SINR intra-frequency measurement is tested by using the parameters in Table A.6.7.11.1.2-2. In all test cases, Cell 1 is the PCell and Cell 2 is the target cell.

Table A.6.7.12.1.2-1: CSI-SINR Intra frequency CSI-SINR supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.12.1.2-2: CSI-SINR Intra frequency test parameters

| Parameter |  |  |  |  | Unit | Test 1 |  |  | Test 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 |  |  | Cell 2 |
| SSB ARFCN |  |  |  |  |  | freq1 |  |  | freq1 |  |  |  |
| Duplex mode |  |  |  | Config 1 |  | FDD |  |  |  |  |  |  |
|  |  |  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |
| TDD configuration |  |  |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |
|  |  |  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |
| DRX Cycle configuration |  |  |  |  | ms | Not Applicable |  |  |  |  |  |  |
| TRS configuration |  |  |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |
|  |  |  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  |  | Config 1 |  | SR.1.1 FDD | - |  | SR.1.1 FDD |  |  | - |
|  |  |  |  | Config 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |  |
|  |  |  |  | Config 3 |  | SR.2.1 TDD |  |  | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  |  | Config 1 |  | CR.1.1 FDD | - |  | CR.1.1 FDD |  |  |  |
|  |  |  |  | Config 2 |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |  |
|  |  |  |  | Config 3 |  | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  |  | Config 1 |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD |  |  | - |
|  |  |  |  | Config 2 |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |  |
|  |  |  |  | Config 3 |  | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  |  |  |  |  | OP.1 |  |  |  |  |  |  |
| CSI-RSSI-Measurement |  |  |  |  |  | Not Applicable |  |  |  |  |  |  |
| Time offset with Cell 1 |  |  | Config 1,2 |  | s | - |  | 2.35 |  | - | 2.35 |  |
|  |  |  | Config 3 |  | s | - |  | 1.17 |  | - | 1.17 |  |
| SSB configuration |  |  |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |
| SMTC configuration |  |  |  | Config 1 |  | SMTC.2 |  |  |  |  |  |  |
|  |  |  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |  |
| CSI-RS for mobility |  |  |  | Config 1 |  | CSI-RS.RRM.FR1.1 FDD |  |  |  |  |  |  |
|  |  |  |  | Config 2 |  | CSI-RS.RRM.FR1.1 TDD |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  | CSI-RS.RRM.FR1.2 TDD |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  |  | Config 1,2 | kHz | 15 |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  | 30 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  | dB | 0 | 0 |  | 0 |  |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -93 |  |  | -116 |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  | -115.5 |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  | -115 |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  | -114.5 |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  | -114 |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  | -113.5 |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  | -113 |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  | -112.5 |  |  |  |
| Note2 | Config 1,2 |  |  |  | dBm/SCS | -93 |  |  | Same as Noc for 15 kHz |  |  |  |
|  | Config 3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -90 |  |  | -113 |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  | -112.5 |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  | -112 |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  | -111.5 |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  | -111 |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  | -110.5 |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  | -110 |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  | -109.5 |  |  |  |
|  |  |  |  |  | dB | 0 | -3.19 |  | -5.46 |  |  | -5.46 |
|  |  |  |  |  | dB | 4.54 | 2.66 |  | -4 |  |  | -4 |
| CSI-RSRPNote3 | Config 1,2 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/SCS | -88.46 | -90.34 |  | -120 |  |  | -120 |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  | -119.5 |  |  | -119.5 |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  | -119 |  |  | -119 |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  | -118.5 |  |  | -118.5 |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  | -118 |  |  | -118 |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  | -117.5 |  |  | -117.5 |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  | -117 |  |  | -117 |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  | -116.5 |  |  | -116.5 |
|  | Config 3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -85.46 | -87.34 |  | -117 |  |  | -117 |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  | -116.5 |  |  | -116.5 |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  | -116 |  |  | -116 |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  | -115.5 |  |  | -115.5 |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  | -115 |  |  | -115 |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  | -114.5 |  |  | -114.5 |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  | -114 |  |  | -114 |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  | -113.5 |  |  | -113.5 |
| CSI-SINR Note3 |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dB | 0 | -3.19 |  | -5.46 |  |  | -5.46 |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |
| IoNote3 |  | Config 1,2 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 9.36MHz | -57.5 |  |  | -85.51 |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  | -85.01 |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  | -84.51 |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  | -84.01 |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  | -83.51 |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  | -83.01 |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  | -82.51 |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  | -82.01 |  |  |  |
|  |  | Config 3 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 38.16MHz | -51.41 |  |  | -79.41 |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  | -78.91 |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  | -78.41 |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  | -77.91 |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  | -77.41 |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  | -76.91 |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  | -76.41 |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  | -75.91 |  |  |  |
| Propagation condition |  |  |  |  | - | AWGN |  |  |  |  |  |  |
| Antenna configuration |  |  |  |  | - | 1x2 |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-SINR, CSI-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: CSI-SINR, CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.7.12.1.3	Test Requirements

The CSI-SINR measurement accuracy shall fulfil the requirements in clause 10.1.12.2.1.

#### A.6.7.12.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.12.2.1	Test Purpose and Environment

The purpose of this test is to verify that the CSI-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.14.2.1 and 10.1.14.2.2.

##### A.6.7.12.2.2	Test Parameters

In this test case the two cells (i.e., Cell 1 and Cell 2) are on different carrier frequencies and measurement gaps are provided. Supported test configurations are shown in Table A.6.7.12.2.2-1. Both absolute accuracy and relative accuracy requirements of CSI-SINR inter-frequency measurement are tested by using test parameters in Table A.6.7.12.2.2-2. In all test cases, Cell 1 is the PCell and Cell 2 is target cell.

Table A.6.7.12.2.2-1: CSI-SINR Inter frequency CSI-SINR supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.12.2.2-2: CSI-SINR Inter frequency test parameters

| Parameter |  |  | Unit | Test 1 |  |  | Test 2 |  |  |  |  | Test 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 |  |  | Cell 2 |  | Cell 1 |  |  | Cell 2 |
| SSB ARFCN |  |  |  | freq1 | freq2 |  | freq1 |  |  | freq2 |  | freq1 |  | freq2 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | - |  | SR.1.1 FDD |  |  | - |  | SR.1.1 FDD |  |  | - |
|  |  | Config 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |  |  | SR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |  |  | SR.2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | - |  | R.1.1 FDD |  |  | - |  | CR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |  |  | CR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |  |  | CR.2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD |  |  | - |  | CCR.1.1 FDD |  |  | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |  |  | CCR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |  |  | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |  |  |  |  |  |
| CSI-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 1,2 | s | - |  | 2.35 |  | - | 2.35 |  | - |  | 2.35 |  |  |
|  |  | Config 3 | s | - |  | 1.17 |  | - | 1.17 |  | - |  | 1.17 |  |  |
| SMTC configuration |  | Config 1 |  | SMTC.2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |  |  |  |  |  |  |
| SSB configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |  |  |  |  |
| CSI-RS for mobility |  | Config 1 |  | CSI-RS.RRM.FR1.1 FDD |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | CSI-RS.RRM.FR1.1 TDD |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | CSI-RS.RRM.FR1.2 TDD |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 |  | 0 |  |  | 0 |  | 0 |  |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -88 |  |  | -108.5 |  |  |  |  | -119.5 |  |  |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  | -119 |  |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  | -118.5 |  |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  | -118 |  |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  | -117.5 |  |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  | -117 |  |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  | -116.5 |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  | -116 |  |  |  |
| Note2 | Config 1,2 N |  | dBm/15kHz | -88 |  |  | -108.5 |  |  |  |  | Same as Noc for 15kHz T |  |  |  |
|  | Config 3 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -85 |  |  | -105.5 |  |  |  |  | -116.5 |  |  |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  | -116 |  |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  | -115.5 |  |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  | -115 |  |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  | -114.5 |  |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  | -114 |  |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  | -114.5 |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  | -113 |  |  |  |
|  |  |  | dB | -1.75 | -1.75 |  | 15 |  |  | 15 |  | -4.0 |  |  | -4.0 |
|  |  |  | dB | -1.75 |  |  | 15 |  |  |  |  | -4.0 |  |  |  |
| CSI-RSRPNote3 | Config 1,2 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/SCS | -89.75 |  |  | -93.5 |  |  |  |  | -123.5 |  |  |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  | -123 |  |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  | -122.5 |  |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  | -122 |  |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  | -121.5 |  |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  | -121 |  |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  | -120.5 |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  | -120 |  |  |  |
|  | Config 3 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -86.75 |  |  | -90.5 |  |  |  |  | -120.5 |  |  |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  | -120 |  |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  | -119.5 |  |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  | -119 |  |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  | -118.5 |  |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  | -118 |  |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  | -117.5 |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  | -117 |  |  |  |
| CSI-SINRNote3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dB | -1.75 |  |  | 15 |  |  |  |  | -4.0 |  |  |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IoNote3 | Config 1,2 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/ 9.36MHz | -57.83 |  |  | -65.4 |  |  |  |  | -90.09 |  |  |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  | -89.59 |  |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  | -89.09 |  |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  | -88.59 |  |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  | -88.09 |  |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  | -87.59 |  |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  | -87.09 |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  | -86.59 |  |  |  |
|  | Config 3 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/ 38.16MHz | -51.73 |  |  | -59.3 |  |  |  |  | -84 |  |  |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  | -83.5 |  |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  | -83 |  |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  | -82.5 |  |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  | -82 |  |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  | -81.5 |  |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  | -81 |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  | -80.5 |  |  |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |  |  |  |  |  |  |
| Antenna configuration |  |  | - | 1x2 |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-SINR, CSI-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: CSI-SINR, CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.7.12.2.3	Test Requirements

The CSI-SINR measurement accuracy shall fulfil the requirements in clause 10.1.14.2.1 and 10.1.14.2.2.

### A.6.7.13	RSTD measurements

#### A.6.7.13.1	RSTD measurement accuracy test case for single positioning frequency layer

##### A.6.7.13.1.1	Test purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the accuracy requirements specified in clause 10.1.23.2 in an environment with AWGN propagation conditions.

The supported test configurations are specified in Table A.6.7.13.1.1-1.

Table A.6.7.13.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cells. Both cells are on the same NR RF channel in FR1. GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. The NR-TDOA-ProvideAssistanceData and NR-TDOA-RequestLocationInformation message as defined in TS 37.355 shall be provided to the UE before the start of the test. The test duration should be larger than the UE measurement period as defined in clause 9.9.2.

Table A.6.7.13.1.1-2: RSTD accuracy test parameters

| Parameter | Config | Unit | Test 1 |  |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |  | Cell 1 | Cell 2 |
| PRS ARFCN | 1~3 |  | freq1 | Freq1 |  | freq1 | Freq1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |
|  | 2 |  | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |
|  | 3 |  | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |
| Duplex mode | 1 |  | FDD |  |  | FDD |  |
|  | 2 |  | TDD |  |  | TDD |  |
|  | 3 |  | TDD |  |  | TDD |  |
| TDD configuration | 1 |  | N/A |  |  | N/A |  |
|  | 2 |  | TDDConf.1.1 |  |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  |  | TDDConf.2.1 |  |
| Measurement gap | 1, 2, 3 |  | GP#24 or GP#0 |  |  | GP#24 or GP#0 |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD | - |  | SR.1.1 FDD | - |
|  | 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 FDD |  |  | SR.2.1 FDD |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD | - |  | CR.1.1 FDD | - |
|  | 2 |  | CR.1.1 TDD | - |  | CR.1.1 TDD | - |
|  | 3 |  | CR.2.1 FDD | - |  | CR.2.1 FDD | - |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD | - |
|  | 2 |  | CCR.1.1 TDD | - |  | CCR.1.1 TDD | - |
|  | 3 |  | CCR.2.1 TDD | - |  | CCR.2.1 TDD | - |
| SSB configuration | 1 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |
|  | 2 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |
|  | 3 |  | SSB.2 FR1 |  |  | SSB.2 FR1 |  |
| OCNG Patterns | 1~3 |  | OP.1 |  |  | OP.1 |  |
| TRS configuration | 1 |  | TRS.1.1 FDD |  | - | TRS.1.1 FDD |  |
|  | 2 |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |
|  | 3 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  |  | DLBWP.1.1 ULBWP.1.1 |  |
| Time offset with Cell 1 | 1 | s | - | 3 |  | - | 3 |
|  | 2,3 |  | - | 3 |  | - | 3 |
| SMTC configuration | 1 |  | SMTC.2 |  |  | SMTC.2 |  |
|  | 2,3 |  | SMTC.1 |  |  | SMTC.1 |  |
| PRS configuration | 1 |  | PRS.1.1 FR1 |  |  | PRS.1.2 FR1 |  |
|  | 2 |  | PRS.1.1 FR1 |  |  | PRS.1.2 FR1 |  |
|  | 3 |  | PRS.2.1 FR1 |  |  | PRS.2.2 FR1 |  |
| PRS Resource slot offset | 1, 2, 3 | slot | 0 | 4 |  | 0 | 4 |
| Expected RSTD | 1, 2, 3 | s | N/A | 3 |  | N/A | 3 |
| Expected RSTD uncertainty | 1, 2, 3 | s | N/A | 5 |  | N/A | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 | 0 |  | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS | 1~3 | dB | 0 | 0 |  | 0 | 0 |
| Note2 | 1,2 | dBm/ SCS | -98 |  |  | -98 |  |
|  | 3 |  | -95 |  |  | -95 |  |
| PRS | 1~3 | dB | -6 | -13 |  | -6 | -13 |
| PRPNote3 | 1,2 | dBm/SCS | -103.7 | -109.9 |  | -103.7 | -109.9 |
|  | 3 |  | -100.7 | -106.9 |  | -100.7 | -106.9 |
| IoNote3 | 1,2 | dBm/ 9.36MHz | -68.80 | -68.80 |  | -68.80 | -68.80 |
|  | 3 | dBm/ 38.16MHz | -62.70 | -62.70 |  | -62.70 | -62.70 |
| PRS | 1~3 | dB | -5.7 | -11.9 |  | -5.7 | -11.9 |
| SSB | 1~3 | dB | -5.7 |  |  | -11.9 |  |
| Propagation condition | 1~3 | - | AWGN |  |  | AWGN |  |
| Antenna configuration | 1~3 |  | 1x2 |  |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the slots with transmitted PRS. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. The Io is calculated based only on the symbols in which PRS is transmitted. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |

##### A.6.7.13.1.2	Test Requirements

The RSTD measurement accuracy for Cell 2 shall fulfil the absolute requirement in clause 10.1.23.2.

#### A.6.7.13.2	RSTD measurement accuracy test case for dual positioning frequency layer

##### A.6.7.13.2.1	Test purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the accuracy requirements specified in clause 10.1.23.2 in an environment with AWGN propagation conditions.

The supported test configurations are specified in Table A.6.7.13.2.1-1.

Table A.6.7.13.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell on NR RF channel #1 in FR1. Cell 2 is a neighbour cell on a different NR RF channel #2 in FR1. GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. The NR-TDOA-ProvideAssistanceData and NR-TDOA-RequestLocationInformation message as defined in TS 37.355 shall be provided to the UE before the start of the test. The test duration should be larger than the UE measurement period as defined in clause 9.9.2.

Table A.6.7.13.2.1-2: RSTD accuracy test parameters

| Parameter | Config | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| PRS ARFCN | 1~3 |  | freq1 | freq2 | freq1 | freq2 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |
|  | 2 |  | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |
|  | 3 |  | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| Duplex mode | 1 |  | FDD |  | FDD |  |
|  | 2 |  | TDD |  | TDD |  |
|  | 3 |  | TDD |  | TDD |  |
| TDD configuration | 1 |  | N/A |  | N/A |  |
|  | 2 |  | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  | TDDConf.2.1 |  |
| Measurement gap | 1, 2, 3 |  | GP#24 or GP#0 |  | GP#24 or GP#0 |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  | 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 FDD |  | SR.2.1 FDD |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD | - | CR.1.1 FDD | - |
|  | 2 |  | CR.1.1 TDD | - | CR.1.1 TDD | - |
|  | 3 |  | CR.2.1 FDD | - | CR.2.1 FDD | - |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  | 2 |  | CCR.1.1 TDD | - | CCR.1.1 TDD | - |
|  | 3 |  | CCR.2.1 TDD | - | CCR.2.1 TDD | - |
| SSB configuration | 1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  | 2 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  | 3 |  | SSB.2 FR1 |  | SSB.2 FR1 |  |
| OCNG Patterns | 1~3 |  | OP.1 |  | OP.1 |  |
| TRS configuration | 1 |  | TRS.1.1 FDD | - | TRS.1.1 FDD |  |
|  | 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |
|  | 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  | DLBWP.1.1 ULBWP.1.1 |  |
| Time offset with Cell 1 | 1 | s | - | 3 | - | 3 |
|  | 2,3 |  | - | 3 | - | 3 |
| SMTC configuration | 1 |  | SMTC.2 |  | SMTC.2 |  |
|  | 2,3 |  | SMTC.1 |  | SMTC.1 |  |
| PRS configuration | 1 |  | PRS.1.1 FR1 |  | PRS.1.2 FR1 |  |
|  | 2 |  | PRS.1.1 FR1 |  | PRS.1.2 FR1 |  |
|  | 3 |  | PRS.2.1 FR1 |  | PRS.2.2 FR1 |  |
| PRS Resource slot offset | 1, 2, 3 | slot | 0 | 4 | 0 | 4 |
| Expected RSTD | 1, 2, 3 | s | N/A | 3 | N/A | 3 |
| Expected RSTD uncertainty | 1, 2, 3 | s | N/A | 5 | N/A | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS | 1~3 | dB | 0 | 0 | 0 | 0 |
| Note2 | 1,2 | dBm/ SCS | -98 |  | -98 |  |
|  | 3 |  | -95 |  | -95 |  |
| PRS | 1~3 | dB | -6 | -13 | -6 | -13 |
| PRPNote3 | 1,2 | dBm/SCS | -104 | -111 | -104 | -111 |
|  | 3 |  | -101 | -108 | -101 | -108 |
| IoNote3 | 1,2 | dBm/ 9.36MHz | -69.07 | -69.83 | -69.07 | -69.83 |
|  | 3 | dBm/ 38.16MHz | -62.98 | -63.74 | -62.98 | -63.74 |
| PRS | 1~3 | dB | -6 | -13 | -6 | -13 |
| Propagation condition | 1~3 | - | AWGN |  | AWGN |  |
| Antenna configuration | 1~3 |  | 1x2 |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the slots with transmitted PRS. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. The Io is calculated based only on the symbols in which PRS is transmitted. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |

##### A.6.7.13.2.2	Test Requirements

The RSTD measurement accuracy for Cell 2 shall fulfil the absolute requirement in clause 10.1.23.2.

#### A.6.7.13.3	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state

##### A.6.7.13.3.1	Test purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the accuracy requirements for reduced number of samples specified in clause 10.1.23.2 in an environment with AWGN propagation conditions. In this test UE that supports supportedDL-PRS-ProcessingSamples is configured by LMF to perform PRS measurement with reduced number of samples.

The supported test configurations are specified in Table A.6.7.13.3.1-1.

Table A.6.7.13.3.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cells. Both cells are on the same NR RF channel in FR1. GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. The NR-TDOA-ProvideAssistanceData and NR-TDOA-RequestLocationInformation message as defined in TS 37.355 shall be provided to the UE before the start of the test. The test duration should be longer than the UE measurement period as defined in clause 9.9.2.7.

Table A.6.7.13.3.1-2: RSTD accuracy test parameters

| Parameter | Config | Unit | Test 1 |  |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |  | Cell 1 | Cell 2 |
| PRS ARFCN | 1~3 |  | freq1 | Freq1 |  | freq1 | Freq1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |
|  | 2 |  | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |
|  | 3 |  | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |
| Duplex mode | 1 |  | FDD |  |  | FDD |  |
|  | 2 |  | TDD |  |  | TDD |  |
|  | 3 |  | TDD |  |  | TDD |  |
| TDD configuration | 1 |  | N/A |  |  | N/A |  |
|  | 2 |  | TDDConf.1.1 |  |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  |  | TDDConf.2.1 |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD | - |  | SR.1.1 FDD | - |
|  | 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 FDD |  |  | SR.2.1 FDD |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD | - |  | CR.1.1 FDD | - |
|  | 2 |  | CR.1.1 TDD | - |  | CR.1.1 TDD | - |
|  | 3 |  | CR.2.1 FDD | - |  | CR.2.1 FDD | - |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD | - |
|  | 2 |  | CCR.1.1 TDD | - |  | CCR.1.1 TDD | - |
|  | 3 |  | CCR.2.1 TDD | - |  | CCR.2.1 TDD | - |
| SSB configuration | 1 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |
|  | 2 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |
|  | 3 |  | SSB.2 FR1 |  |  | SSB.2 FR1 |  |
| OCNG Patterns | 1~3 |  | OP.1 |  |  | OP.1 |  |
| TRS configuration | 1 |  | TRS.1.1 FDD |  | - | TRS.1.1 FDD |  |
|  | 2 |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |
|  | 3 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  |  | DLBWP.1.1 ULBWP.1.1 |  |
| Time offset with Cell 1 | 1 | s | - | 3 |  | - | 3 |
|  | 2,3 |  | - | 3 |  | - | 3 |
| SMTC configuration | 1 |  | SMTC.2 |  |  | SMTC.2 |  |
|  | 2,3 |  | SMTC.1 |  |  | SMTC.1 |  |
| PRS configuration | 1 |  | PRS.1.4 FR1 |  |  | PRS.1.4 FR1 |  |
|  | 2 |  | PRS.1.4 FR1 |  |  | PRS.1.4 FR1 |  |
|  | 3 |  | PRS.2.4 FR1 |  |  | PRS.2.4 FR1 |  |
| PRS BW | 1,2,3 |  | 48 PRBs |  |  |  |  |
| PRS muting info | 1~3 |  | ‘10’ | ‘01’ |  | ‘10’ | ‘01’ |
| Expected RSTD | 1, 2, 3 | s | N/A | 3 |  | N/A | 3 |
| Expected RSTD uncertainty | 1, 2, 3 | s | N/A | 5 |  | N/A | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 | 0 |  | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |
| Note2 | 1,2 | dBm/ SCS | -98 |  |  | -98 |  |
|  | 3 |  | -95 |  |  | -95 |  |
|  | 1~3 | dB | -3 | 5 |  | -3 | 5 |
| PRS-RSRPNote3 | 1,2 | dBm/SCS | -101 | -93 |  | -101 | -93 |
|  | 3 |  | -98 | -90 |  | -98 | -90 |
| SS-RSRPNote3 | 1,2 | dBm/SCS | -88 | -88 |  | -88 | -88 |
|  | 3 |  | -85 | -85 |  | -85 | -85 |
| IoNote3 | 1,2 | dBm/ 9.36MHz | -69.5 | -67.9 |  | -69.5 | -67.9 |
|  | 3 | dBm/ 38.16MHz | -63.4 | -61.8 |  | -63.4 | -61.8 |
|  | 1~3 | dB | -3 | 5 |  | -3 | 5 |
| Propagation condition | 1~3 | - | AWGN |  |  | AWGN |  |
| Antenna configuration | 1~3 |  | 1x2 |  |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |

##### A.6.7.13.3.2	Test Requirements

The RSTD measurement accuracy for Cell 2 shall fulfil the absolute requirement in clause 10.1.23.2.

#### A.6.7.13.4	RSTD measurement accuracy test case with Rx TEG

##### A.6.7.13.4.1	Test purpose and Environment

The purpose of the test is to verify that the RSTD measurement when the measurements of reference cell and neighbor cell are within the same Rx TEG meets the accuracy requirements specified in clause 10.1.23.2 in an environment with AWGN propagation conditions.

The supported test configurations are specified in Table A.6.7.13.4.1-1.

Table A.6.7.13.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR1. GP#24 is configured if UE supports GP#24, otherwise GP#0 is configured. The NR-TDOA-ProvideAssistanceData and NR-TDOA-RequestLocationInformation message as defined in TS 37.355 shall be provided to the UE before the start of the test. The test duration should be larger than the UE measurement period as defined in clause 9.9.2.

The UE is requested to provide the Rx TEG in the test via nr-UE-RxTEG-Request-r17 in NR-TDOA-RequestLocationInformation.

The test applies to the UE supporting Rx TEG defiend in NR-UE-TEG-Capability and reporting the same Rx TEG for the measurements of reference cell and neighbour cell.

Table A.6.7.13.4.1-2: RSTD accuracy test parameters

| Parameter | Config | Unit | Test 1 |  |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |  | Cell 1 | Cell 2 |
| PRS ARFCN | 1~3 |  | freq1 | Freq1 |  | freq1 | Freq1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |
|  | 2 |  | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |
|  | 3 |  | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |
| Duplex mode | 1 |  | FDD |  |  | FDD |  |
|  | 2 |  | TDD |  |  | TDD |  |
|  | 3 |  | TDD |  |  | TDD |  |
| TDD configuration | 1 |  | N/A |  |  | N/A |  |
|  | 2 |  | TDDConf.1.1 |  |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  |  | TDDConf.2.1 |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD | - |  | SR.1.1 FDD | - |
|  | 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 FDD |  |  | SR.2.1 FDD |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD | - |  | CR.1.1 FDD | - |
|  | 2 |  | CR.1.1 TDD | - |  | CR.1.1 TDD | - |
|  | 3 |  | CR.2.1 FDD | - |  | CR.2.1 FDD | - |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD | - |
|  | 2 |  | CCR.1.1 TDD | - |  | CCR.1.1 TDD | - |
|  | 3 |  | CCR.2.1 TDD | - |  | CCR.2.1 TDD | - |
| SSB configuration | 1 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |
|  | 2 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |
|  | 3 |  | SSB.2 FR1 |  |  | SSB.2 FR1 |  |
| OCNG Patterns | 1~3 |  | OP.1 |  |  | OP.1 |  |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |  |
| TRS configuration | 1 |  | TRS.1.1 FDD |  | - | TRS.1.1 FDD |  |
|  | 2 |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |
|  | 3 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  |  | DLBWP.1.1 ULBWP.1.1 |  |
| Time offset with Cell 1 | 1 | s | - | 3 |  | - | 3 |
|  | 2,3 |  | - | 3 |  | - | 3 |
| SMTC configuration | 1 |  | SMTC.2 |  |  | SMTC.2 |  |
|  | 2,3 |  | SMTC.1 |  |  | SMTC.1 |  |
| PRS configuration | 1 |  | PRS.1.1 FR1 |  |  | PRS.1.2 FR1 |  |
|  | 2 |  | PRS.1.1 FR1 |  |  | PRS.1.2 FR1 |  |
|  | 3 |  | PRS.2.1 FR1 |  |  | PRS.2.2 FR1 |  |
| PRS Resource slot offset | 1~3 | slot | 0 | 4 |  | 0 | 4 |
| Expected RSTD | 1, 2, 3 | s | N/A | 3 |  | N/A | 3 |
| Expected RSTD uncertainty | 1, 2, 3 | s | N/A | 5 |  | N/A | 5 |
| Note2 | 1,2 | dBm/ SCS | -98 |  |  | -98 |  |
|  | 3 |  | -95 |  |  | -95 |  |
| PRS | 1~3 | dB | -5.33 | -12.19 |  | -5.33 | -12.19 |
| PRPNote3 | 1,2 | dBm/SCS | -103 | -109 |  | -103 | -109 |
|  | 3 |  | -100 | -106 |  | -100 | -106 |
| IoNote3 | 1,2 | dBm/ 9.36MHz | -68.60 | -68.60 |  | -68.60 | -68.60 |
|  | 3 | dBm/ 38.16MHz | -62.51 | -62.51 |  | -62.51 | -62.51 |
| PRS | 1~3 | dB | -5 | -11 |  | -5 | -11 |
| Propagation condition | 1~3 | - | AWGN |  |  | AWGN |  |
| Antenna configuration | 1~3 |  | 1x2 |  |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |

##### A.6.7.13.4.2	Test Requirements

The RSTD measurement for Cell 1 and Cell 2 should fulfil the absolute accuracy requirements with same Rx TEG in clause 10.1.23.2.

### A.6.7.14	PRS-RSRP measurements

#### A.6.7.14.1	SA: measurement accuracy with PRS in FR1

##### A.6.7.14.1.1	Test Purpose and Environment

The purpose of this test is to verify that the PRS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.24.2.1 and 10.1.24.2.2.

##### A.6.7.14.1.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Supported test configurations are shown in table A.6.7.14.1.2-1. Both absolute and relative accuracy of PRS-RSRP measurements are tested by using the parameters in A.6.7.14.1.2-2. In all test cases, Cell 1 is the PCell.

Table A.6.7.14.1.2-1: PRS-RSRP supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table A.6.7.14.1.2-2: PRS-RSRP test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |
| Cell ID |  |  |  | 489 | 0 | 489 | 0 |  |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| BWP BW |  | Config 1 |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD | NA | TRS.1.1 FDD | NA |  |
|  |  | Config 2 |  | TRS.1.1 TDD | NA | TRS.1.1 TDD | NA |  |
|  |  | Config 3 |  | TRS.1.2 TDD | NA | TRS.1.2 TDD | NA |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |
| Measurement gap |  |  |  | GP#24 or GP#0 Note 7 |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |  |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  | SR2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | - | CR.1.1 FDD | - |  |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  | CR2.1 TDD |  |  |
| Control channel RMC |  | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |  |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |
|  |  | Config 3 |  | CCR2.1 TDD |  | CCR2.1 TDD |  |  |
| PRS configuration |  | Config 1 |  | PRS.1.3 FR1 | PRS.1.3 FR1 | PRS.1.4 FR1 | PRS.1.4 FR1 |  |
|  |  | Config 2 |  | PRS.1.3 FR1 | PRS.1.3 FR1 | PRS.1.4 FR1 | PRS.1.4 FR1 |  |
|  |  | Config 3 |  | PRS.2.3 FR1 | PRS.2.3 FR1 | PRS.2.4 FR1 | PRS.2.4 FR1 |  |
| PRS Resource slot offset (slot) |  | Config 1,2,3 | slot | 0 | 4 | 0 | 4 |  |
| SSB configuration |  | Config 1 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |  |
|  |  | Config 2 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |  |
|  |  | Config 3 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |  |
| Time offset with Cell 1 |  | Config 1 | s | - | 3 | - | 3 |  |
|  |  | Config 2,3 |  | - | 3 | - | 3 |  |
| Expected RSTD |  | Config 1,2,3 | s | 3 |  |  |  |  |
| Expected RSTD uncertainty |  | Config 1,2,3 | s | 5 |  |  |  |  |
| SMTC configuration |  | Config 1 |  | SMTC.2 |  |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  | dB | 0 | 0 | 0 | 0 |  |
| Note2 | Config 1,2 |  | dBm/15KhZ | -98 |  | -98 |  |  |
|  | Config 3 |  |  | -98 |  | -98 |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  | -98 |  |  |
|  | Config 3 |  |  | -95 |  | -95 |  |  |
| PRS |  |  | dB | -2.41 | -12.12 | -2.41 |  | -12.12 |
| PRS |  |  | dB | -2 | -10 | -2 |  | -10 |
| PRP Note3 | Config 1, 2 |  | dBm/SCS | -100 | -108 | -100 |  | -108 |
|  | Config 3 |  |  | -97 | -105 | -97 |  | -105 |
| IoNote3 | Config 1,2 |  | dBm/9.36MHz | -67.67 |  | -67.67 |  |  |
|  | Config 3 |  | dBm/38.16MHz | -61.57 |  | -61.57 |  |  |
| Propagation condition |  |  |  | AWGN |  |  |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: PRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Void. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification Note 7: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |  |  |  |  |

##### A.6.7.14.1.3	Test Requirements

In each test, the absolute PRS-RSRP measurement for each cell shall fulfil the absolute accuracy requirement in clause 10.1.24.2.1. The relative PRS-RSRP measurement between the two PRS resources within the same cell shall fulfil the relative accuracy requirement in clause 10.1.24.2.2.

#### A.6.7.14.2	SA: measurement accuracy with PRS in FR1 with reduced sample number

##### A.6.7.14.2.1	Test Purpose and Environment

The purpose of this test is to verify that the accuracy of PRS-RSRP measurement with reduced sample number is within the specified limits provided that PRS is transmitted within the active BWP of the UE. UE can support supportedDL-PRS-ProcessingSamples-RRC-CONNECTED, and the LMF indicates the UE to perform positioning measurements with reduced number of samples NsampleNsample via reducedDL-PRS-ProcessingSamples. This test will verify the requirements in clauses 10.1.24.2.1 and 10.1.24.2.2.

##### A.6.7.14.2.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Supported test configurations are shown in table A.6.7.14.2.2-1. Both absolute and relative accuracy of PRS-RSRP measurements are tested by using the parameters in A.6.7.14.2.2-2. In all test cases, Cell 1 is the PCell.

Table A.6.7.14.2.2-1: PRS-RSRP supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table A.6.7.14.2.2-2: PRS-RSRP test parameters

| Parameter |  |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |
| Cell ID |  |  |  | 489 | 0 |
| SSB ARFCN |  |  |  | freq1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |
|  |  | Config 2,3 |  | TDD |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |
| BWP BW |  | Config 1 |  | 10: NRB,c = 52 |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD | NA |
|  |  | Config 2 |  | TRS.1.1 TDD | NA |
|  |  | Config 3 |  | TRS.1.2 TDD | NA |
| DRX Cycle |  |  | ms | Not Applicable |  |
| Measurement gap |  |  |  | GP#24 or GP#0 Note 7 |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | - |
|  |  | Config 2 |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | - |
|  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR2.1 TDD |  |
| Control channel RMC |  | Config 1 |  | CCR.1.1 FDD | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR2.1 TDD |  |
| PRS configuration |  | Config 1 |  | PRS.1.4 FR1 | PRS.1.4 FR1 |
|  |  | Config 2 |  | PRS.1.4 FR1 | PRS.1.4 FR1 |
|  |  | Config 3 |  | PRS.2.4 FR1 | PRS.2.4 FR1 |
| PRS BW |  | Config 1 | RB | 52 | 52 |
|  |  | Config 2 |  | 52 | 52 |
|  |  | Config 3 |  | 106 | 106 |
| PRS Resource slot offset (slot) |  | Config 1,2,3 | slot | 0 | 4 |
| SSB configuration |  | Config 1 |  | SSB.1 FR1 | SSB.1 FR1 |
|  |  | Config 2 |  | SSB.1 FR1 | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.2 FR1 | SSB.2 FR1 |
| Time offset with Cell 1 |  | Config 1 | ms | - | 3 |
|  |  | Config 2,3 | s | - | 3 |
| SMTC configuration |  | Config 1 |  | SMTC.2 |  |
|  |  | Config 2,3 |  | SMTC.1 |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |
|  |  | Config 3 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | dBm/15KhZ | -106 |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H |  | Not applicableNote 5 |  |
| Note2 | Config 1,2 |  | dBm/SCS | -106 |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H |  | Not applicableNote 5 |  |
|  |  |  | dB | -6 | -6 |
|  |  |  | dB | -6 | -6 |
| SS-RSRP |  |  | dB | Same as PRS-RSRP |  |
| PRS-RSRP Note3 | Config 1, 2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | dBm/SCS | -112 | -112 |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H |  | Not applicable Note 5 | Not applicable Note 5 |
| IoNote3 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | dBm/9.36MHz | -77.07 |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | dBm/38.16MHz | Not applicable Note 5 |  |
| Propagation condition |  |  |  | AWGN |  |
| Antenna configuration |  |  |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Subtest 1 is not used when testing with 30kHz SSB SCS. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification Note 7: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |  |

##### A.6.7.14.2.3	Test Requirements

In the test, the absolute PRS-RSRP measurement for each cell shall fulfil the absolute accuracy requirement in clause 10.1.24.2.1. The relative PRS-RSRP measurement between the two PRS resources within the same cell shall fulfil the relative accuracy requirement in clause 10.1.24.2.2.

#### A.6.7.14.3	Void

##### A.6.7.14.3.1	Void

##### A.6.7.14.3.2	Void

##### A.6.7.14.3.3	Void

### A.6.7.15	UE Rx-Tx time difference measurements

#### A.6.7.15.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA

##### A.6.7.15.1.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx time difference measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.25.2. The test is conducted in AWGN propagation condition in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations in listed in Table A.6.7.15.1.1-1.

Table A.6.7.15.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). Both cells are on the same RF channel in FR1.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE before the start of the test.

The UE is configured with measurement gap pattern ID #0 or ID #24 before the test.

The UE is configured to transmit positioning SRS on Cell 1 during the test.

The test equipment measures the transmit timing of the UE using the transmitted SRS and measures the receive timing using the PRS. The test equipment then compares the difference of these two timings to the UE Rx-Tx measurement reported by the UE for each cell.

##### A.6.7.15.1.2	Test parameters

The UE Rx-Tx time difference accuracy test parameters are given in Table A.6.7.15.1.2-1.

Table A.6.7.15.1.2-1: UE Rx-Tx time difference measurement accuracy test parameters

| Parameter | Unit | Test configuration | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| RF Channel Number |  | 1,2,3 | 1 | 1 | 1 | 1 |
| Measurement gap |  | 1,2,3 | GP#24 or GP#0 Note 4 |  | GP#24 or GP#0 Note 4 |  |
| DRX |  | 1,2,3 | OFF |  | OFF |  |
| Time offset with Cell 1 | s | 1, 2, 3 | N/A | 3 | N/A | 3 |
| TDD configuration |  | 1 | N/A | N/A | N/A | N/A |
|  |  | 2 | TDDConf.1.1 | TDDConf.1.1 | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 | TDDConf.2.1 | TDDConf.2.1 | TDDConf.2.1 | TDDConf.2.1 |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD | N/A | SR.1.1 FDD | N/A |
|  |  | 2 | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD | N/A | CR.1.1 FDD | N/A |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD | N/A | CCR.1.1 FDD | N/A |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 | OP.1 | OP.1 | OP.1 |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD | N/A | TRS.1.1 FDD | N/A |
|  |  | 2 | TRS.1.1 TDD |  | TRS.1.1 TDD |  |
|  |  | 3 | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 | N/A | DLBWP.0.1 ULBWP.0.1 | N/A |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 | N/A | DLBWP.1.1 | N/A |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 | N/A | ULBWP.1.1 | N/A |
| PRS configuration |  | 1 | PRS.1.1 FR1 | PRS.1.1 FR1 | PRS.1.2 FR1 | PRS.1.2 FR1 |
|  |  | 2 | PRS.1.1 FR1 | PRS.1.1 FR1 | PRS.1.2 FR1 | PRS.1.2 FR1 |
|  |  | 3 | PRS.2.1 FR1 | PRS.2.1 FR1 | PRS.2.2 FR1 | PRS.2.2 FR1 |
| PRS Resource slot offset | slot | 1, 2, 3 | 0 | 4 | 0 | 4 |
| SRS configuration |  | 1 | POS-SRS.1 | N/A | POS-SRS.1 | N/A |
|  |  | 2 | POS-SRS.1 | N/A | POS-SRS.1 | N/A |
|  |  | 3 | POS-SRS.2 | N/A | POS-SRS.2 | N/A |
| Note 2 | dBm/SCS | 1 | -98 |  | -98 |  |
|  |  | 2 | -98 |  | -98 |  |
|  |  | 3 | -95 |  | -95 |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  | -98 |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -2.41 | -12.12 | -2.41 | -12.12 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -2 | -10 | -2 | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRP Note 3 | dBm/SCS kHz | 1 | -100 | -108 | -100 | -108 |
|  |  | 2 | -100 | -108 | -100 | -108 |
|  |  | 3 | -97 | -105 | -97 | -105 |
| Io | dBm/9.36 MHz | 1 | -67.67 | -67.67 | -67.67 | -67.67 |
|  | dBm/9.36 MHz | 2 | -67.67 | -67.67 | -67.67 | -67.67 |
|  | dBm/38.16 MHz | 3 | -61.57 | -61.57 | -61.57 | -61.57 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  | AWGN |  |
| Note 1: Void. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |  |  |

Table A.6.7.15.1.2-2: Void

##### A.6.7.15.1.3	Test requirements

The UE Rx-Tx time difference measurement fulfils the UE Rx-Tx measurement accuracy requirements specified in clause 10.1.25.2 for both Cell 1 and Cell 2.

#### A.6.7.15.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR1 SA

##### A.6.7.15.2.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx time difference measurement accuracy with reduced number of samples is within the specified limits. This test will verify the requirements in clause 10.1.25.2. The test is conducted in AWGN propagation condition in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations in listed in Table A.6.7.15.2.1-1.

Table A.6.7.15.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). All cells are on the same RF channel in FR1.

The NR-Multi-RTT-ProvideAssistanceData and NR-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE before the start of the test.

The UE is configured to measure UE Rx-Tx time difference using reduced number of samples via requestedDL-PRS-ProcessingSamples in NR-Multi-RTT-RequestLocationInformation during the test.

The UE is configured with measurement gap pattern ID #0 or ID #24 before the test.

The UE is configured to transmit positioning SRS on Cell 1 during the test.

The test equipment measures the transmit timing of the UE using the transmitted SRS and measures the receive timing using the PRS. The test equipment then compares the difference of these two timings to the UE Rx-Tx measurement reported by the UE for each cell.

##### A.6.7.15.2.2	Test parameters

The UE Rx-Tx time difference accuracy test parameters are given in Table A.6.7.15.2.2-1.

Table A.6.7.15.2.2-1: UE Rx-Tx time difference measurement accuracy test parameters

| Parameter | Unit | Test configuration | Test 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |
| RF Channel Number |  | 1,2,3 | 1 | 1 |
| Measurement gap |  | 1,2,3 | GP#24 or GP#0 Note 4 |  |
| DRX |  | 1,2,3 | OFF |  |
| Time offset with Cell 1 | s | 1, 2, 3 | N/A | 3 |
| TDD configuration |  | 1 | N/A | N/A |
|  |  | 2 | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 | TDDConf.2.1 | TDDConf.2.1 |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD | N/A |
|  |  | 2 | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD | N/A |
|  |  | 2 | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD | N/A |
|  |  | 2 | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 | OP.1 |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD | N/A |
|  |  | 2 | TRS.1.1 TDD |  |
|  |  | 3 | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 | N/A |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 | N/A |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 | N/A |
| PRS configuration |  | 1 | PRS.1.1 FR1 | PRS.1.1 FR1 |
|  |  | 2 | PRS.1.1 FR1 | PRS.1.1 FR1 |
|  |  | 3 | PRS.2.1 FR1 | PRS.2.1 FR1 |
| PRS BW |  | 1 | 48 PRBs | 48 PRBs |
|  |  | 2 | 48 PRBs | 48 PRBs |
|  |  | 3 | 48 PRBs | 48 PRBs |
| PRS Resource slot offset | slot | 1, 2, 3 | 0 | 4 |
| SRS configuration |  | 1 | POS-SRS.1 | N/A |
|  |  | 2 | POS-SRS.1 | N/A |
|  |  | 3 | POS-SRS.2 | N/A |
| Note 2 | dBm/SCS | 1 | -98 |  |
|  |  | 2 | -98 |  |
|  |  | 3 | -95 |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRS | dB | 1 | -1.76 | -6.01 |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRS | dB | 1 | 0 | -3 |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRP Note 3 | dBm/SCS kHz | 1 | -98 | -101 |
|  |  | 2 | -98 | -101 |
|  |  | 3 | -95 | -98 |
| Io | dBm/9.36 MHz | 1 | -66.07 | -66.07 |
|  | dBm/9.36 MHz | 2 | -66.07 | -66.07 |
|  | dBm/38.16 MHz | 3 | -59.97 | -59.97 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

##### A.6.7.15.2.3	Test requirements

The UE Rx-Tx time difference measurement with reduced number of samples fulfils the UE Rx-Tx measurement accuracy.

#### A.6.7.15.3	UE Rx-Tx time difference measurement accuracy with RxTx TEG

##### A.6.7.15.3.1	Test purpose and environment

The purpose of the test is to verify that the relative UE Rx-Tx time difference measurement accuracy when the two measurements are within the same RxTx TEG is within the specified limits. This test will verify the requirements in clause 10.1.25.x. The test is conducted in AWGN propagation condition in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations is listed in Table A.6.7.15.3.1-1.

Table A.6.7.15.3.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). Both cells are on the same RF channel in FR1.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE before the start of the test.

The UE is requested to provide the RxTx TEG in the test via nr-UE-RxTxTEG-Request-r17 in NR-Multi-RTT-RequestLocationInformation.

The test applies to the UE supporting RxTx TEG defiend in NR-UE-TEG-Capability and reporting the same RxTx TEG for the two UE Rx-Tx measurements.

The UE is configured with measurement gap pattern ID #0 or ID #24 before the test.

The UE is configured to transmit positioning SRS on Cell 1 during the test.

The test equipment measures the transmit timing of the UE using the transmitted SRS and measures the receive timing using the PRS. The UE Rx-Tx time difference is derived by the difference of the receiving timing and the transmit timing for each cell.

##### A.6.7.15.3.2	Test parameters

The UE Rx-Tx time difference accuracy test parameters are given in Table A.6.7.15.3.2-1. y requirements specified in clause 10.1.25.2 for both Cell 1 and Cell 2.

Table A.6.7.15.3.2-2: UE Rx-Tx time difference measurement accuracy test parameters

| Parameter | Unit | Test configuration | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| RF Channel Number |  | 1,2,3 | 1 | 1 | 1 | 1 |
| Measurement gap |  | 1,2,3 | GP#24 or GP#0 Note 4 |  | GP#24 or GP#0 Note 4 |  |
| DRX |  | 1,2,3 | OFF |  | OFF |  |
| Time offset with Cell 1 | s | 1, 2, 3 | N/A | 3 | N/A | 3 |
| TDD configuration |  | 1 | N/A | N/A | N/A | N/A |
|  |  | 2 | TDDConf.1.1 | TDDConf.1.1 | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 | TDDConf.2.1 | TDDConf.2.1 | TDDConf.2.1 | TDDConf.2.1 |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD | N/A | SR.1.1 FDD | N/A |
|  |  | 2 | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD | N/A | CR.1.1 FDD | N/A |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD | N/A | CCR.1.1 FDD | N/A |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 | OP.1 | OP.1 | OP.1 |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD | N/A | TRS.1.1 FDD | N/A |
|  |  | 2 | TRS.1.1 TDD |  | TRS.1.1 TDD |  |
|  |  | 3 | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 | N/A | DLBWP.0.1 ULBWP.0.1 | N/A |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 | N/A | DLBWP.1.1 | N/A |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 | N/A | ULBWP.1.1 | N/A |
| PRS configuration |  | 1 | PRS.1.1 FR1 | PRS.1.1 FR1 | PRS.1.2 FR1 | PRS.1.2 FR1 |
|  |  | 2 | PRS.1.1 FR1 | PRS.1.1 FR1 | PRS.1.2 FR1 | PRS.1.2 FR1 |
|  |  | 3 | PRS.2.1 FR1 | PRS.2.1 FR1 | PRS.2.2 FR1 | PRS.2.2 FR1 |
| PRS Resource slot offset | slot | 1, 2, 3 | 0 | 4 | 0 | 4 |
| SRS configuration |  | 1 | POS-SRS.1 | N/A | POS-SRS.1 | N/A |
|  |  | 2 | POS-SRS.1 | N/A | POS-SRS.1 | N/A |
|  |  | 3 | POS-SRS.2 | N/A | POS-SRS.2 | N/A |
| Note 2 | dBm/SCS | 1 | -98 |  | -98 |  |
|  |  | 2 | -98 |  | -98 |  |
|  |  | 3 | -95 |  | -95 |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  | -98 |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -5.33 | -12.19 | -5.33 | -12.19 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -5 | -11 | -5 | -11 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRP Note 3 | dBm/SCS kHz | 1 | -103 | -109 | -103 | -109 |
|  |  | 2 | -103 | -109 | -103 | -109 |
|  |  | 3 | -100 | -106 | -100 | -106 |
| Io | dBm/9.36 MHz | 1 | -68.60 | -68.60 | -68.60 | -68.60 |
|  | dBm/9.36 MHz | 2 | -68.60 | -68.60 | -68.60 | -68.60 |
|  | dBm/38.16 MHz | 3 | -62.51 | -62.51 | -62.51 | -62.51 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  | AWGN |  |
| Note 1: Void. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |  |  |

##### A.6.7.15.3.3	Test requirements

The relative accuracy is derived by the difference of the UE Rx-Tx measurements on the two cells.

The UE Rx-Tx time difference measurements for Cell 1 and Cell 2 fulfil the relative UE Rx-Tx measurement accuracy requirements specified in clause 10.1.25.x.

### A.6.7.16	PRS-RSRPP measurements

#### A.6.7.16.1	SA: measurement accuracy with PRS in FR1

##### A.6.7.16.1.1	Test Purpose and Environment

The purpose of this test is to verify that the accuracy of PRS-RSRPP measurement in RRC_CONNECTED is within the specified limits. This test will verify the requirements in clauses 10.1.X.y and 10.1. X.z.

##### A.6.7.16.1.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Supported test configurations are shown in table A.6.7.16.1.2-1. Both absolute and relative accuracy of PRS-RSRPP measurements are tested by using the parameters in A.6.7.16.1.2-2. In all test cases, Cell 1 is the PCell.

Table A.6.7.16.1.2-1: PRS-RSRPP supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table A.6.7.16.1.2-2: PRS-RSRPP test parameters

| Parameter |  |  | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 |  | Cell 2 | Cell 1 | Cell 2 |  |
| Cell ID |  |  |  | 489 |  | 0 | 489 | 0 |  |
| SSB ARFCN |  |  |  | freq1 |  |  | freq1 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW |  | Config 1 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  | NA | TRS.1.1 FDD | NA |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  | NA | TRS.1.1 TDD | NA |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  | NA | TRS.1.2 TDD | NA |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| Measurement gap |  |  |  | GP#24 or GP#0 Note 7 |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  | - | SR.1.1 FDD | - |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  | SR2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  | - | CR.1.1 FDD | - |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  | CR2.1 TDD |  |  |
| Control channel RMC |  | Config 1 |  | CCR.1.1 FDD |  | - | CCR.1.1 FDD | - |  |
|  |  | Config 2 |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | Config 3 |  | CCR2.1 TDD |  |  | CCR2.1 TDD |  |  |
| PRS configuration |  | Config 1 |  | PRS.1.3 FR1 |  | PRS.1.3 FR1 | PRS.1.4 FR1 | PRS.1.4 FR1 |  |
|  |  | Config 2 |  | PRS.1.3 FR1 |  | PRS.1.3 FR1 | PRS.1.4 FR1 | PRS.1.4 FR1 |  |
|  |  | Config 3 |  | PRS.2.3 FR1 |  | PRS.2.3 FR1 | PRS.2.4 FR1 | PRS.2.4 FR1 |  |
| PRS Resource slot offset (slot) |  | Config 1,2,3 | slot | 0 |  | 4 | 0 | 4 |  |
| SSB configuration |  | Config 1 |  | SSB.1 FR1 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |  |
|  |  | Config 2 |  | SSB.1 FR1 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |  |
|  |  | Config 3 |  | SSB.2 FR1 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |  |
| Time offset with Cell 1 |  | Config 1 | ms | - |  | 3 | - | 3 |  |
|  |  | Config 2,3 | s | - |  | 3 | - | 3 |  |
| SMTC configuration |  | Config 1 |  | SMTC.2 |  |  |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  | 0 | 0 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | dBm/15KhZ | -106 |  |  | -88 |  |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H |  | Not applicableNote 5 |  |  | -94 |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -106 |  |  | -88 |  |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H |  | Not applicableNote 5 |  |  | -91 |  |  |
|  |  |  | dB | 2.46 | -5.97 |  | 2.46 |  | -5.97 |
|  |  |  | dB | 6 | 1 |  | 6 |  | 1 |
| PRS-RSRP Note3 | Config 1, 2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | dBm/SCS | -100 |  | -105 | -82 |  | -87 |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H |  | Not applicable Note 5 |  | Not applicable Note 5 | -85 |  | -90 |
| IoNote3 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | dBm/9.36MHz | -70.09 |  |  | -52.09 |  |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | dBm/38.16MHz | Not applicable Note 5 |  |  | -51.99 |  |  |
| Propagation condition |  |  |  | Two-tap channel defined in 38.101-4 Annex B.2.4, a = 1, τd=0.45 µs and fD=5 Hz |  |  |  |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: PRS-RSRPP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Subtest 1 is not used when testing with 30kHz SSB SCS. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification Note 7: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |  |  |  |  |  |

##### A.6.7.16.1.3	Test Requirements

In each test, the absolute PRS-RSRPP measurement for each cell shall fulfil the absolute accuracy requirement in clause 10.1.X.y. The relative PRS-RSRPP measurement between the two PRS resources within the same cell shall fulfil the relative accuracy requirement in clause 10.1.X.z.

#### A.6.7.16.2	SA: measurement accuracy with reduced PRS samples in FR1

##### A.6.7.16.2.1	Test Purpose and Environment

The purpose of this test is to verify that the PRS-RSRPP measurement accuracy with Nsample = 1 in FR1 is within the specified limits. This test will verify the requirements in clauses [10.1.24.Z.1] and [10.1.24.Z.2].

The UE under test should support [supportedDL-PRS-ProcessingSamples ], and the TE indicates the UE to perform positioning measurements with reduced number of samples. The PRS bandwidth is contained within the active BWP and the power difference between the serving cell SS-RSRP and neighbour cell PRS-RSRP is within 6dB, so that Nsample= 1 is assumed.

##### A.6.7.16.2.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Supported test configurations are shown in table A.6.7.16.2.2-1. Both absolute and relative accuracy of PRS-RSRPP measurements are tested by using the parameters in A.6.7.16.2.2-2. In all test cases, Cell 1 is the PCell.

Table A.6.7.16.2.2-1: PRS-RSRPP supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table A.6.7.16.2.2-2: PRS-RSRPP test parameters

| Parameter |  |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |  |
| Cell ID |  |  |  | 489 | 0 |  |
| SSB ARFCN |  |  |  | freq1 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |
|  |  | Config 2,3 |  | TDD |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |
| BWP BW |  | Config 1 |  | 10: NRB,c = 52 |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  | NA |
|  |  | Config 2 |  | TRS.1.1 TDD |  | NA |
|  |  | Config 3 |  | TRS.1.2 TDD |  | NA |
| DRX Cycle |  |  | ms | Not Applicable |  |  |
| Measurement gap |  |  |  | GP#24 or GP#0 Note 5 |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  | - |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  | - |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |
| Control channel RMC |  | Config 1 |  | CCR.1.1 FDD |  | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  |  |
|  |  | Config 3 |  | CCR2.1 TDD |  |  |
| PRS configuration |  | Config 1 |  | PRS.1.4 FR1 |  | PRS.1.4 FR1 |
|  |  | Config 2 |  | PRS.1.4 FR1 |  | PRS.1.4 FR1 |
|  |  | Config 3 |  | PRS.2.4 FR1 |  | PRS.2.4 FR1 |
| PRS bandwidth |  | Config 1,2,3 |  | 48 PRBs |  | 48 PRBs |
| PRS Resource slot offset (slot) |  | Config 1,2,3 | slot | 0 |  | 4 |
| SSB configuration |  | Config 1 |  | SSB.1 FR1 |  | SSB.1 FR1 |
|  |  | Config 2 |  | SSB.1 FR1 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.2 FR1 |  | SSB.2 FR1 |
| Time offset with Cell 1 |  | Config 1 | ms | - | 3 |  |
|  |  | Config 2,3 | s | - | 3 |  |
| SMTC configuration |  | Config 1 |  | SMTC.2 |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/15KhZ | -98 |  |  |
|  | Config 3 |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |
|  | Config 3 |  |  | -95 |  |  |
|  |  |  | dB | 2.46 |  | -5.97 |
|  |  |  | dB | 6 |  | 1 |
| PRS-RSRP Note3 | Config 1, 2 |  | dBm/SCS | -92 |  | -97 |
|  | Config 3 |  |  | -89 |  | -94 |
| SS-RSRP Note3 | Config 1, 2 |  | dBm/SCS | -92 |  | -97 |
|  | Config 3 |  |  | -89 |  | -94 |
| IoNote3 | Config 1,2 |  | dBm/9.36MHz | -70.09 |  |  |
|  | Config 3 |  | dBm/38.16MHz | -63.99 |  |  |
| Propagation condition |  |  |  | [Two-tap channel] Note6 |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. Note 6: The two-tap channel model is defined in 38.101-4 Annex B.2.4 (a = 1, τd=0.45 µs and fD=5 Hz). |  |  |  |  |  |  |

##### A.6.7.16.2.3	Test Requirements

In each test, the absolute PRS-RSRPP measurement for each cell shall fulfil the absolute accuracy requirement in clause [10.1.24.Z.1]. The relative PRS-RSRPP measurement between the two PRS resources within the same cell shall fulfil the relative accuracy requirement in clause [10.1.24.Z.2].

## A.6.8	Measurement procedure in RRC_INACTIVE

### A.6.8.1	RSTD measurements

#### A.6.8.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_INACTIVE state

##### A.6.8.1.1.1	Test Purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the requirements specified in Clause 5.6.2.5 in an environment with AWGN propagation conditions in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations are specified in Table A.6.8.1.1.1-1.

Table A.6.8.1.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All 3 cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. During time duration T1, the UE shall be in RRC_CONNECTED state and shall not have any timing information of Cell 2 and Cell 3. During T2 UE shall be in RRC_INACTIVE state and all three cells transmit PRS resources within initial DL BWP of the UE and with the same numerology as the initial DL BWP.

Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The NR-DL-TDOA-ProvideAssistanceData and nr-DL-TDOA-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-TDOA assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the first DRX cycle containing a DL PRS resource(s).

The UE is configured with DRX cycle of 1.28s.

The general test parameters are listed in Table A.6.8.1.1.1-2, and cell specific test parameters are listed in Table A.6.8.1.1.1-3 and Table A.6.8.1.1.1-4.

Table A.6.8.1.1.1-2: General test parameters for RSTD measurement reporting delay

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  |  | Cell 1 | Reference cell is the cell in the DL-TDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 38.215 [4] and TS 37.355 [34]. The reference cell is the PCell in this test case. |
| Neighbor cells |  |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at the first and second places in the neighbour cell list in the DL-TDOA assistance data. |
| SSB configuration | Config 1 |  | SSB.1 FR1 |  |
|  | Config 2 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB.2 FR1 |  |
| SMTC configuration | Config 1 |  | SMTC.2 |  |
|  | Config 2 |  | SMTC.1 |  |
|  | Config 3 |  | SMTC.1 |  |
| PDSCH RMC configuration | Config 1 |  | SR.1.1 FDD |  |
|  | Config 2 |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration | Config 1 |  | CR.1.1 FDD | As specified in clause A.3.1.2.1 |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration | Config 1 |  | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Initial BWP configuration | Config 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active UL BWP configuration | Config 1,2,3 |  | ULBWP.1.1 |  |
| PRS Configuration | Config 1 |  | PRS.1.1 FR1 | As specified in clause A.3.31 |
|  | Config 2 |  | PRS.1.2 FR1 |  |
|  | Config 3 |  | PRS.2.1 FR1 |  |
| Physical cell ID PCI |  |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  |  | Normal |  |
| DRX |  | s | 1.28 |  |
| Radio frame receive time offset between the cells at the UE antenna connector |  | s | Cell 2 to Cell 1: 0 Cell 3 to Cell 1: 3 | PRS are transmitted from synchronous cells |
| Expected RSTD |  | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355 [34] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells |  | s | 5 | The corresponding parameter in the DL-TDOA assistance ta specified in TS 37.355 [34] is the expectedRSTD-Uncertainty index |
| Number of cells provided in DL-TDOA assistance data |  |  | 16 | Including the reference cell |
| PRS muting info |  |  | Cell 1: ‘10’ Cell 2: ‘01’ Cell 3: ‘10’ | Correponds to prs-MutingInfo defined in TS 37.355 [34] |
| PRS resource RE offset |  |  | Cell 1: 0 Cell 2: 0 Cell 3: 1 | Cell 1 and Cell 3 are configured with different resource offsets |
| T1 |  | s | 3 | The length of the time interval from the beginning of each test |
| T2 |  | s | 5 | The length of the time interval that follows immediately after time interval T1. |

Table A.6.8.1.1.1-3: Cell-specific test parameters for RSTD measurement reporting delay during T1

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Positiong frequency layer |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | N/A | N/A |
| Note 3 | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 | dBm/SCS | -98 |  |  |
|  | Config 3 | dBm/SCS | -95 |  |  |
| PRS |  | dB | -Infinity | -Infinity | -Infinity |
| SSB |  | dB | 10 | -Infinity | -Infinity |
| Io Note 4 | Config 1 | dBm/ 9.36MHz | -68.63 | -70.05 | -70.05 |
|  | Config 2 | dBm/ 9.36MHz | -68.63 | -70.05 | -70.05 |
|  | Config 3 | dBm/ 38.16MHz | -63.20 | -63.96 | -63.96 |
| SSB RP Note4 | Config 1 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 2 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 3 | dBm/SCS | -88 | -Infinity | -Infinity |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned after the end of time period T2 to UEs that do not support SDT for measurement reporting. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |

Table A.6.8.1.1.1-4: Cell-specific test parameters for RSTD measurement reporting delay during T2

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T2 | T2 | T2 |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | OP.1 | OP.1 |
| PRACH configuration |  |  | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 |
| Note 3 | Config 1 | dBm/SCS | -98 | -98 | -98 |
|  | Config 2 | dBm/SCS | -98 | -98 | -98 |
|  | Config 3 | dBm/SCS | -95 | -95 | -95 |
| PRS | Config 1 | dB | -5.45 | -11.67 | -11.67 |
|  | Config 2 | dB | -5.45 | -11.67 | -11.67 |
|  | Config 3 | dB | -5.45 | -11.67 | -11.67 |
| Io Note 4 | Config 1 | dBm/ 9.36MHz | -69.59 | -69.93 | -69.93 |
|  | Config 2 | dBm/ 96.48MHz | -69.59 | -69.93 | -69.93 |
|  | Config 3 | dBm/ 38.16MHz | -63.72 | -63.89 | -63.89 |
| PRS |  | dB | -6 | -13 | -13 |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T2) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned after the end of time period T2 to UEs that do not support SDT for measurement reporting. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |  |

##### A.6.8.1.1.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in Clause 5.6.2.5.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the DL-TDOA assistance data, Cell 1, within the time duration specified in section 5.6.2.5 starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 10.1.23.3, i.e., between RSTD_0000000 and RSTD1970049.

#### A.6.8.1.2		NR RSTD measurement reporting delay test case with reduced number of samples in RRC_INACTIVE, FR1 SA

##### A.6.8.1.2.1	Test Purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the requirements specified in Clause 5.6.2 in an environment with AWGN propagation conditions in FR1 in standalone scenario when single-sample measurements are requested by the LMF. This test is applicable to UEs that support [FG 14-2 PRS measurement for reduced sample in RRC_inactive state].

The supported test configurations are specified in Table A.6.8.1.2.1-1.

Table A.6.8.1.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All 3 cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. The UE shall be in RRC_CONNECTED state during T1 and in RRC_INACTIVE state during T2. During time duration T1, the UE shall not have any timing information of Cell 2 and Cell 3. All three cells transmit PRS during T2. PRS resources from all three cells are transmitted within the initial DL BWP of the UE and with the same numerology as the initial DL BWP.

Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The beginning of the time interval T2 shall be aligned with the beginning of the first DRX cycle in RRC_INACTIVE.

The NR-DL-TDOA-ProvideAssistanceData and nr-DL-TDOA-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-TDOA assistance data and location information request.

The nr-DL-TDOA-RequestLocationInformation IE should indicate to the UE that single-sample measurements are requested, i.e. requestedDL-PRS-ProcessingSamples-r17 is set to m1.

The general test parameters are listed in Table A.6.8.1.2.1-2, and cell specific test parameters are listed in Table A.6.8.1.2.1-3 and Table A.6.8.1.2.1-4.

Table A.6.8.1.2.1-2: General test parameters for RSTD measurement reporting delay

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  |  | Cell 1 | Reference cell is the cell in the DL-TDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 38.215 [4] and TS 37.355 [34]. The reference cell is the PCell in this test case. |
| Neighbor cells |  |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at the first and second places in the neighbour cell list in the DL-TDOA assistance data. |
| SSB configuration | Config 1 |  | SSB.1 FR1 |  |
|  | Config 2 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB.2 FR1 |  |
| SMTC configuration | Config 1 |  | SMTC.2 |  |
|  | Config 2 |  | SMTC.1 |  |
|  | Config 3 |  | SMTC.1 |  |
| PDSCH RMC configuration | Config 1 |  | SR.1.1 FDD |  |
|  | Config 2 |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration | Config 1 |  | CR.1.1 FDD | As specified in clause A.3.1.2.1 |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration | Config 1 |  | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Initial BWP configuration | Config 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |
| PRS Configuration | Config 1 |  | PRS.1.2 FR1 | As specified in clause A.3.31. The number of PRS RBs is the same as for the channel BW. |
|  | Config 2 |  | PRS.1.2 FR1 |  |
|  | Config 3 |  | PRS.2.2 FR1 |  |
| PRS Resource slot offset | Config 1, 2, 3 | slots | Cell 1, 2, 3: 0 |  |
| Physical cell ID PCI |  |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  |  | Normal |  |
| DRX cycle length |  | s | 1.28 |  |
| Radio frame receive time offset between the cells at the UE antenna connector |  | s | Cell 2 to Cell 1: 0 Cell 3 to Cell 1: 3 | PRS are transmitted from synchronous cells |
| Expected RSTD |  | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355 [34] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells |  | s | 5 | The corresponding parameter in the DL-TDOA assistance ta specified in TS 37.355 [34] is the expectedRSTD-Uncertainty index |
| Number of cells provided in DL-TDOA assistance data |  |  | 16 | Including the reference cell |
| PRS muting info |  |  | Cell 1: ‘10’ Cell 2: ‘01’ Cell 3: ‘10’ | Correponds to prs-MutingInfo defined in TS 37.355 [34] |
| PRS resource RE offset |  |  | Cell 1: 0 Cell 2: 0 Cell 3: 1 | Cell 1 and Cell 3 are configured with different resource offsets |
| T1 |  | s | 3 | The length of the time interval from the beginning of each test |
| T2 |  | s | [5] | The length of the time interval that follows immediately after time interval T1 |

Table A.6.8.1.2.1-3: Cell-specific test parameters for RSTD measurement reporting delay during T1

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Positiong frequency layer |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | N/A | N/A |
| Note 3 | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 | dBm/SCS | -98 |  |  |
|  | Config 3 | dBm/SCS | -95 |  |  |
| PRS |  | dB | -Infinity | -Infinity | -Infinity |
| SSB |  | dB | 0 | -Infinity | -Infinity |
| Io Note 4 | Config 1 | dBm/ 9.36MHz | -67.05 | -67.05 | -67.05 |
|  | Config 2 | dBm/ 9.36MHz | -67.05 | -67.05 | -67.05 |
|  | Config 3 | dBm/ 38.16MHz | -60.94 | -60.94 | -60.94 |
| SSB RP Note4 | Config 1 | dBm/SCS | -98 | -Infinity | -Infinity |
|  | Config 2 | dBm/SCS | -98 | -Infinity | -Infinity |
|  | Config 3 | dBm/SCS | -95 | -Infinity | -Infinity |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |

Table A.6.8.1.2.1-4: Cell-specific test parameters for RSTD measurement reporting delay during T2

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T2 | T2 | T2 |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | OP.1 | OP.1 |
| PRACH configuration |  |  | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 |
| Note 3 | Config 1 | dBm/SCS | -98 | -98 | -98 |
|  | Config 2 | dBm/SCS | -98 | -98 | -98 |
|  | Config 3 | dBm/SCS | -95 | -95 | -95 |
| PRS | Config 1 | dB | 0 | -3 | -3 |
|  | Config 2 | dB | 0 | -3 | -3 |
|  | Config 3 | dB | 0 | -3 | -3 |
| Io Note 4 | Config 1 | dBm/ 9.36MHz | -66.07 | -68.29 | -66.07 |
|  | Config 2 | dBm/ 9.36MHz | -66.07 | -68.29 | -66.07 |
|  | Config 3 | dBm/ 38.16MHz | -61.52 | -62.99 | -61.52 |
| PRS |  | dB | 0 | -3 | -3 |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that all active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols, except for slots where PRS is transmitted by any of the cells. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels apply only for symbols where PRS is transmitted. |  |  |  |  |  |

##### A.6.8.1.2.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in Clause 5.6.2.5.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the DL-TDOA assistance data, Cell 1, within the time duration specified in section 5.6.2.5 starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 10.1.23.3, i.e., between RSTD_0000000 and RSTD_1970049.

### A.6.8.2	PRS-RSRP measurements

#### A.6.8.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE

##### A.6.8.2.1.1	Test purpose and Environment

The purpose of the test is to verify that the PRS-RSRP measurement in RRC_INACTIVE meets the delay requirements specified in clause 5.6.3.5 in an environment with AWGN propagation conditions.

The supported test configurations are specified in Table A.6.8.2.1.1-1.

Table A.6.8.2.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR1. The test consists of two consecutive time intervals, with duration of T1 and T2. Both cells transmit PRS during T2.

During T1 UE is in RRC_CONNECTED, the NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n. In the next DL slot after slot #n, UE is released into RRC_INACTIVE.

The beginning of the time interval T2 is the first PRS resource occasion occurring T after the slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The general test parameters are listed in Table A.6.8.2.1.1-2, and cell specific test parameters are listed in Table A.6.8.2.1.1-3.

Table A.6.8.2.1.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |
|  |  | 2 | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | 1.28s |  |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | [6] |  |

Table A.6.8.2.1.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 2 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 3 | PRS.2.4 FR1 |  | PRS.2.4 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -101 | -Infinity | -108 |
|  |  | 2 | -Infinity | -101 | -Infinity | -108 |
|  |  | 3 | -Infinity | -98 | -Infinity | -105 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -88 | -88 | -88 | -88 |
|  |  | 2 | -88 | -88 | -88 | -88 |
|  |  | 3 | -85 | -85 | -85 | -85 |
| Io | dBm/9.36 MHz | 1 | N/A | -62.25 | N/A | -62.25 |
|  | dBm/9.36 MHz | 2 |  | -62.25 |  | -62.25 |
|  | dBm/38.16 MHz | 3 |  | -56.16 |  | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP/PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.8.2.1.2	Test Requirements

The UE shall perform and report the PRS-RSRP measurements for Cell 1 and Cell 2, within the time limit specified in clause 5.6.3.5, starting from the beginning of time interval T2.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.8.2.2	PRS-RSRP reporting delay test case with reduced number of samples in RRC_INACTIVE

##### A.6.8.2.2.1	Test purpose and Environment

The purpose of the test is to verify that the PRS-RSRP measurement meets the delay requirements specified in clause 5.6.3.5 in an environment with AWGN propagation conditions when single-sample measurements are requested by the LMF. This test is applicable to UEs that support [FG 14-2 PRS measurement for reduced sample in RRC_inactive state].

The supported test configurations are specified in Table A.6. 8.2.2.1-1.

Table A.6. 8.2.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. The UE shall be in RRC_CONNECTED state during T1 and in RRC_INACTIVE state during T2. During time duration T1, the UE shall not have any timing information of Cell 2. Both cells transmit PRS during T2. PRS resources from both cells are transmitted within the initial DL BWP of the UE and with the same numerology as the initial DL BWP.

The beginning of the time interval T2 shall be aligned with the beginning of the first DRX cycle in RRC_INACTIVE.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-AoD assistance data and location information request.

The nr-DL-AoD-RequestLocationInformation IE should indicate to the UE that single-sample measurements are requested, i.e. requestedDL-PRS-ProcessingSamples-r17 is set to m1.

The general test parameters are listed in Table A.6.8.2.2.1-2, and cell specific test parameters are listed in Table A.6.8.2.2.1-3.

Table A.6.8.2.2.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |
|  |  | 2 | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX cycle length | s | 1, 2, 3 | 1.28 |  |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 3 |  |
| T2 | s | 1, 2, 3 | [5] |  |

Table A.6.8.2.2.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| PRS configuration Note 5 |  | 1 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 2 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 3 | PRS.2.2 FR1 |  | PRS.2.2 FR1 |  |
| PRS Resource slot offset | slots | 1, 2, 3 | 0 |  | 0 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | 0 | -Infinity | -3 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | 0 | -Infinity | -3 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -98 | -Infinity | -101 |
|  |  | 2 | -Infinity | -98 | -Infinity | -101 |
|  |  | 3 | -Infinity | -95 | -Infinity | -98 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -98 | -98 | -98 | -98 |
|  |  | 2 | -98 | -98 | -98 | -98 |
|  |  | 3 | -95 | -95 | -95 | -95 |
| Io Note 3 | dBm/9.36 MHz | 1 | N/A | -67.04 | N/A | -68.29 |
|  | dBm/9.36 MHz | 2 |  | -67.04 |  | -68.29 |
|  | dBm/38.16 MHz | 3 |  | -62.19 |  | -62.99 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP/PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io levels apply only for symbols where PRS is transmitted. Note 4: OCNG shall be used such that all active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols, except for slots where PRS is transmitted by any of the cells. Note 5: The number of PRS RBs is the same as for the channel BW. |  |  |  |  |  |  |

##### A.6.8.2.2.2	Test Requirements

The UE shall perform and report the PRS-RSRP measurements for Cell 1 and Cell 2, within the time limit specified in clause 5.6.3.5, starting from the beginning of time interval T2.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.6.8.3	UE Rx-Tx time difference measurements

#### A.6.8.3.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA

##### A.6.8.3.1.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx measurement in RRC_INACTIVE state meets the requirements specified in clause 5.6.4 in AWGN propagation condition in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations in listed in Table A.6.8.3.1.1-1.

Table A.6.8.3.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). Both cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. Cell 1 and Cell 2 mute PRS transmission during T1 and transmit PRS during T2.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n. In the next DL slot after slot #n, UE is released into RRC_INACTIVE.

The beginning of the time interval T2 is the first PRS resource occasion occurring T after the slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The UE is configured to transmit positioning SRS during T2.

The general test parameters and cell specific test parameters are as given in Table A.6.8.3.1.1-2 and Table A.6.8.3.1.1-3 respectively.

Table A.6.8.3.1.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| RF Channel Number |  | 1, 2, 3 | 1 | For both Cell 1 and Cell 2 |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |
|  |  | 2 | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX cycle |  | 1, 2, 3 | 1.28s |  |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | [10] |  |

Table A.6.8.3.1.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 2 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 3 | PRS.2.2 FR1 |  | PRS.2.2 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| SRS configuration |  | 1 | POS-SRS.1 |  | N/A |  |
|  |  | 2 | POS-SRS.1 |  | N/A |  |
|  |  | 3 | POS-SRS.2 |  | N/A |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2.41 | -Infinity | -12.12 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -100 | -Infinity | -108 |
|  |  | 2 | -Infinity | -100 | -Infinity | -108 |
|  |  | 3 | -Infinity | -97 | -Infinity | -105 |
| Io | dBm/9.36 MHz | 1 | N/A | -67.67 | N/A | -67.67 |
|  | dBm/9.36 MHz | 2 |  | -67.67 |  | -67.67 |
|  | dBm/38.16 MHz | 3 |  | -61.57 |  | -61.57 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |

##### A.6.8.3.1.2	Test requirements

The UE Rx-Tx time difference measurement time in RRC_INACTIVE state fulfils the requirements specified in clause 5.6.4.

The UE shall perform and report the UE Rx-Tx time difference measurements for Cell 1 and Cell 2 within the specified UE Rx-Tx time difference measurement time starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause 10.1.25.3.1.

#### A.6.8.3.2	UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE, FR1 SA

##### A.6.8.3.2.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx measurement meets the requirements specified in clause 5.6.4.5 in AWGN propagation condition in FR1 in standalone scenario when single-sample measurements are requested by the LMF. This test is applicable to UEs that support [FG 14-2 PRS measurement for reduced sample in RRC_inactive state].

The supported test configurations in listed in Table A.6.8.3.2.1-1.

Table A.6.8.3.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). All cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. The UE shall be in RRC_CONNECTED state during T1 and in RRC_INACTIVE state during T2. During time duration T1, the UE shall not have any timing information of Cell 2. Both cells transmit PRS during T2. PRS resources from both cells are transmitted within the initial DL BWP of the UE and with the same numerology as the initial DL BWP.

The beginning of the time interval T2 shall be aligned with the beginning of the first DRX cycle in RRC_INACTIVE.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the multi-RTT assistance data and location information request.

The nr-Multi-RTT-RequestLocationInformation IE should indicate to the UE that single-sample measurements are requested, i.e. requestedDL-PRS-ProcessingSamples-r17 is set to m1.

The UE is configured to transmit SRS during T2.

The general test parameters and cell specific test parameters are as given in Table A.6.8.3.2.1-2 and Table A.6.8.3.2.1-3 respectively.

Table A.6.8.3.2.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| RF Channel Number |  | 1, 2, 3 | 1 | For both Cell 1 and Cell 2 |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |
|  |  | 2 | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX cycle length | s | 1, 2, 3 | 1.28 |  |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| T1 | s | 1, 2, 3 | 3 |  |
| T2 | s | 1, 2, 3 | 5 |  |

Table A.6.8.3.2.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| PRS configuration Note 5 |  | 1 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 2 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 3 | PRS.2.2 FR1 |  | PRS.2.2 FR1 |  |
| PRS Resource slot offset | slots | 1, 2, 3 | 0 |  | 0 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | 0 | -Infinity | -3 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | 0 | -Infinity | -3 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -98 | -Infinity | -101 |
|  |  | 2 | -Infinity | -98 | -Infinity | -101 |
|  |  | 3 | -Infinity | -95 | -Infinity | -98 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -98 | -98 | -98 | -98 |
|  |  | 2 | -98 | -98 | -98 | -98 |
|  |  | 3 | -95 | -95 | -95 | -95 |
| Io Note 3 | dBm/9.36 MHz | 1 | N/A | -67.04 | N/A | -68.29 |
|  | dBm/9.36 MHz | 2 |  | -67.04 |  | -68.29 |
|  | dBm/38.16 MHz | 3 |  | -62.19 |  | -62.99 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP/PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io levels apply only for symbols where PRS is transmitted. Note 4: OCNG shall be used such that all active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols, except for slots where PRS is transmitted by any of the cells. Note 5: The number of PRS RBs is the same as for the channel BW. |  |  |  |  |  |  |

##### A.6.8.3.2.2	Test requirements

The UE Rx-Tx time difference measurement time fulfils the requirements specified in clause 5.6.4.5.

The UE shall perform and report the UE Rx-Tx time difference measurements for Cell 1 and Cell 2 within the specified UE Rx-Tx time difference measurement time starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause 10.1.25.3.1.

### A.6.8.4	PRS-RSRPP measurements

#### A.6.8.4.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state

##### A.6.8.4.1.1	Test purpose and Environment

The purpose of the test is to verify that the PRS-RSRPP measurement meets the delay requirements specified in clause 5.6.5.5 in an environment with AWGN propagation conditions.

The supported test configurations are specified in Table A.6.8.4.1.1-1.

Table A.6.8.4.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

The test consists of two consecutive time intervals, with duration of T1 and T2. During time duration T1, the UE shall be in RRC_CONNECTED state and shall not have any timing information of Cell 2. During T2 UE shall be in RRC_INACTIVE state and all both cells transmit PRS resources within initial DL BWP of the UE and with the same numerology as the initial DL BWP.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first DRX cycle containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The general test parameters are listed in Table A.6.8.4.1.1-2, and cell specific test parameters are listed in Table A.6.8.4.1.1-3.

Table A.6.8.4.1.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |
|  |  | 2 | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX | s | 1, 2, 3 | 1.28 | ON |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | [5] |  |

Table A.6.8.4.1.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 2 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 3 | PRS.2.4 FR1 |  | PRS.2.4 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -101 | -Infinity | -108 |
|  |  | 2 | -Infinity | -101 | -Infinity | -108 |
|  |  | 3 | -Infinity | -98 | -Infinity | -105 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -88 | -88 | -88 | -88 |
|  |  | 2 | -88 | -88 | -88 | -88 |
|  |  | 3 | -85 | -85 | -85 | -85 |
| Io | dBm/9.36 MHz | 1 | N/A | -62.25 | N/A | -62.25 |
|  | dBm/9.36 MHz | 2 |  | -62.25 |  | -62.25 |
|  | dBm/38.16 MHz | 3 |  | -56.16 |  | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned after the end of time period T2 to UEs that do not support SDT for measurement reporting. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP/PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.8.4.1.2	Test Requirements

The UE shall perform and report the PRS-RSRPP measurements for Cell 1 and Cell 2, within the time limit specified in clause 5.6.5.5, starting from the beginning of time interval T2.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.8.4.2	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state for reduced number of samples

##### A.6.8.4.2.1	Test purpose and Environment

The purpose of the test is to verify that the PRS-RSRPP measurement meets the delay requirements specified in clause 5.6.5.5 in an environment with AWGN propagation conditions for reduced number of samples. In this test UE that supports supportedDL-PRS-ProcessingSamples is configured by LMF to perform PRS measurement with reduced number of samples.

The supported test configurations are specified in Table A.6.8.4.2.1-1.

Table A.6.8.4.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

The test consists of two consecutive time intervals, with duration of T1 and T2. During time duration T1, the UE shall be in RRC_CONNECTED state and shall not have any timing information of Cell 2. During T2 UE shall be in RRC_INACTIVE state and both cells transmit PRS resources within initial DL BWP of the UE and with the same numerology as the initial DL BWP.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first DRX cycle containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The general test parameters are listed in Table A.6.8.4.2.1-2, and cell specific test parameters are listed in Table A.6.8.4.2.1-3.

Table A.6.8.4.2.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |
|  |  | 2 | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX | s | 1, 2, 3 | 1.28 | ON |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | [5] |  |

Table A.6.8.4.2.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 2 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 3 | PRS.2.4 FR1 |  | PRS.2.4 FR1 |  |
| PRS BW |  | 1,2,3 | 48 PRBs |  | 48PRBs |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | 5 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | 5 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -101 | -Infinity | -93 |
|  |  | 2 | -Infinity | -101 | -Infinity | -93 |
|  |  | 3 | -Infinity | -98 | -Infinity | -90 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -88 | -88 | -88 | -88 |
|  |  | 2 | -88 | -88 | -88 | -88 |
|  |  | 3 | -85 | -85 | -85 | -85 |
| Io | dBm/9.36 MHz | 1 | N/A | -69.5 | N/A | -67.9 |
|  | dBm/9.36 MHz | 2 |  | -69.5 |  | -67.9 |
|  | dBm/38.16 MHz | 3 |  | -63.4 |  | -61.8 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned after the end of time period T2 to UEs that do not support SDT for measurement reporting. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP/PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.8.4.2.2	Test Requirements

The UE shall perform and report the PRS-RSRPP measurements for Cell 1 and Cell 2, within the time limit specified in clause 5.6.5.5, starting from the beginning of time interval T2.

The rate of correct events observed during repeated tests shall be at least 90%.

## A.6.9	Measurement performance requirements in RRC_INACTIVE

### A.6.9.1	RSTD measurements

#### A.6.9.1.1	RSTD measurement accuracy test case for single positioning frequency layer in FR1 in RRC_INACTIVE state

##### A.6.9.1.1.1	Test purpose and Environment

The purpose of the test is to verify that the RSTD measurement in RRC_INACTIVE state meets the accuracy requirements specified in clause 10.1.23.2 in an environment with AWGN propagation conditions.

The supported test configurations are specified in Table A.6.9.1.1.1-1.

Table A.6.9.1.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR1. The UE is configured with DRX cycle of 1.28s. The NR-TDOA-ProvideAssistanceData and NR-TDOA-RequestLocationInformation message as defined in TS 37.355 shall be provided to the UE before the start of the test. The test duration should be larger than the UE measurement period as defined in clause 5.6.2.5.

Table A.6.9.1.1.1-2: RSTD accuracy test parameters

| Parameter | Config | Unit | Test 1 |  |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |  | Cell 1 | Cell 2 |
| PRS ARFCN | 1~3 |  | freq1 | Freq1 |  | freq1 | Freq1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |
|  | 2 |  | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |
|  | 3 |  | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |
| Duplex mode | 1 |  | FDD |  |  | FDD |  |
|  | 2 |  | TDD |  |  | TDD |  |
|  | 3 |  | TDD |  |  | TDD |  |
| TDD configuration | 1 |  | N/A |  |  | N/A |  |
|  | 2 |  | TDDConf.1.1 |  |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  |  | TDDConf.2.1 |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD | - |  | SR.1.1 FDD | - |
|  | 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 FDD |  |  | SR.2.1 FDD |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD | - |  | CR.1.1 FDD | - |
|  | 2 |  | CR.1.1 TDD | - |  | CR.1.1 TDD | - |
|  | 3 |  | CR.2.1 FDD | - |  | CR.2.1 FDD | - |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD | - |
|  | 2 |  | CCR.1.1 TDD | - |  | CCR.1.1 TDD | - |
|  | 3 |  | CCR.2.1 TDD | - |  | CCR.2.1 TDD | - |
| SSB configuration | 1 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |
|  | 2 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |
|  | 3 |  | SSB.2 FR1 |  |  | SSB.2 FR1 |  |
| OCNG Patterns | 1~3 |  | OP.1 |  |  | OP.1 |  |
| TRS configuration | 1 |  | TRS.1.1 FDD |  | - | TRS.1.1 FDD |  |
|  | 2 |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |
|  | 3 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |
| Time offset with Cell 1 | 1 | s | - | 3 |  | - | 3 |
|  | 2,3 |  | - | 3 |  | - | 3 |
| SMTC configuration | 1 |  | SMTC.2 |  |  | SMTC.2 |  |
|  | 2,3 |  | SMTC.1 |  |  | SMTC.1 |  |
| PRS configuration | 1 |  | PRS.1.4 FR1 |  |  | PRS.1.4 FR1 |  |
|  | 2 |  | PRS.1.4 FR1 |  |  | PRS.1.4 FR1 |  |
|  | 3 |  | PRS.2.4 FR1 |  |  | PRS.2.4 FR1 |  |
| PRS muting info | 1~3 |  | ‘10’ | ‘01’ |  | ‘10’ | ‘01’ |
| Expected RSTD | 1, 2, 3 | s | N/A | 3 |  | N/A | 3 |
| Expected RSTD uncertainty | 1, 2, 3 | s | N/A | 5 |  | N/A | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 | 0 |  | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |
| Note2 | 1,2 | dBm/ SCS | -98 |  |  | -98 |  |
|  | 3 |  | -95 |  |  | -95 |  |
|  | 1~3 | dB | -6 | -13 |  | -6 | -13 |
| PRS-RSRPNote3 | 1,2 | dBm/SCS | -104 | -111 |  | -104 | -111 |
|  | 3 |  | -101 | -108 |  | -101 | -108 |
| IoNote3 | 1,2 | dBm/ 9.36MHz | -69.07 | -69.83 |  | -69.07 | -69.83 |
|  | 3 | dBm/ 38.16MHz | -62.98 | -63.74 |  | -62.98 | -63.74 |
|  | 1~3 | dB | -6 | -13 |  | -6 | -13 |
| Propagation condition | 1~3 | - | AWGN |  |  | AWGN |  |
| Antenna configuration | 1~3 |  | 1x2 |  |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |

##### A.6.9.1.1.2	Test Requirements

The RSTD measurement accuracy for Cell 2 shall fulfil the absolute requirement in clause 10.1.23.2.

#### A.6.9.1.2	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_INACTIVE state

##### A.6.9.1.2.1	Test purpose and Environment

The purpose of the test is to verify that the RSTD measurement in RRC_INACTIVE state meets the accuracy requirements specified in clause 10.1.23.2 in an environment with AWGN propagation conditions. In this test UE that supports supportedDL-PRS-ProcessingSamples is configured by LMF to perform PRS measurement with reduced number of samples.

The supported test configurations are specified in Table A.6.9.1.2.1-1.

Table A.6.9.1.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR1. The UE is configured with DRX cycle of 1.28s. The NR-TDOA-ProvideAssistanceData and NR-TDOA-RequestLocationInformation message as defined in TS 37.355 shall be provided to the UE before the start of the test. The test duration should be larger than the UE measurement period as defined in clause 5.6.2.5.

Table A.6.9.1.2.1-2: RSTD accuracy test parameters

| Parameter | Config | Unit | Test 1 |  |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |  | Cell 1 | Cell 2 |
| PRS ARFCN | 1~3 |  | freq1 | Freq1 |  | freq1 | Freq1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |
|  | 2 |  | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |
|  | 3 |  | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |
| Duplex mode | 1 |  | FDD |  |  | FDD |  |
|  | 2 |  | TDD |  |  | TDD |  |
|  | 3 |  | TDD |  |  | TDD |  |
| TDD configuration | 1 |  | N/A |  |  | N/A |  |
|  | 2 |  | TDDConf.1.1 |  |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  |  | TDDConf.2.1 |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD | - |  | SR.1.1 FDD | - |
|  | 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 FDD |  |  | SR.2.1 FDD |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD | - |  | CR.1.1 FDD | - |
|  | 2 |  | CR.1.1 TDD | - |  | CR.1.1 TDD | - |
|  | 3 |  | CR.2.1 FDD | - |  | CR.2.1 FDD | - |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD | - |
|  | 2 |  | CCR.1.1 TDD | - |  | CCR.1.1 TDD | - |
|  | 3 |  | CCR.2.1 TDD | - |  | CCR.2.1 TDD | - |
| SSB configuration | 1 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |
|  | 2 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |
|  | 3 |  | SSB.2 FR1 |  |  | SSB.2 FR1 |  |
| OCNG Patterns | 1~3 |  | OP.1 |  |  | OP.1 |  |
| TRS configuration | 1 |  | TRS.1.1 FDD |  | - | TRS.1.1 FDD |  |
|  | 2 |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |
|  | 3 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  |  | - |  |
| Time offset with Cell 1 | 1 | s | - | 3 |  | - | 3 |
|  | 2,3 |  | - | 3 |  | - | 3 |
| SMTC configuration | 1 |  | SMTC.2 |  |  | SMTC.2 |  |
|  | 2,3 |  | SMTC.1 |  |  | SMTC.1 |  |
| PRS configuration | 1 |  | PRS.1.4 FR1 |  |  | PRS.1.4 FR1 |  |
|  | 2 |  | PRS.1.4 FR1 |  |  | PRS.1.4 FR1 |  |
|  | 3 |  | PRS.2.4 FR1 |  |  | PRS.2.4 FR1 |  |
| PRS BW | 1,2,3 |  | 48 PRBs |  |  |  |  |
| PRS muting info | 1~3 |  | ‘10’ | ‘01’ |  | ‘10’ | ‘01’ |
| Expected RSTD | 1, 2, 3 | s | N/A | 3 |  | N/A | 3 |
| Expected RSTD uncertainty | 1, 2, 3 | s | N/A | 5 |  | N/A | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 | 0 |  | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |
| Note2 | 1,2 | dBm/ SCS | -98 |  |  | -98 |  |
|  | 3 |  | -95 |  |  | -95 |  |
|  | 1~3 | dB | -3 | 5 |  | -3 | 5 |
| PRS-RSRPNote3 | 1,2 | dBm/SCS | -101 | -93 |  | -101 | -93 |
|  | 3 |  | -98 | -90 |  | -98 | -90 |
| SS-RSRPNote3 | 1,2 | dBm/SCS | -88 | -88 |  | -88 | -88 |
|  | 3 |  | -85 | -85 |  | -85 | -85 |
| IoNote3 | 1,2 | dBm/ 9.36MHz | -69.5 | -67.9 |  | -69.5 | -67.9 |
|  | 3 | dBm/ 38.16MHz | -63.4 | -61.8 |  | -63.4 | -61.8 |
|  | 1~3 | dB | -3 | 5 |  | -3 | 5 |
| Propagation condition | 1~3 | - | AWGN |  |  | AWGN |  |
| Antenna configuration | 1~3 |  | 1x2 |  |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |

##### A.6.9.1.2.2	Test Requirements

The RSTD measurement accuracy for Cell 2 shall fulfil the absolute requirement in clause 10.1.23.2.

### A.6.9.2	PRS-RSRP measurements

#### A.6.9.2.1	SA: measurement accuracy with PRS in FR1 in RRC_INACTIVE

##### A.6.9.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the PRS-RSRP measurement accuracy in RRC_INACTIVE is within the specified limits. This test will verify the requirements in clauses 10.1.24.2.1 and 10.1.24.2.2.

##### A.6.9.2.1.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Supported test configurations are shown in table A.6.9.2.1.2-1. Both absolute and relative accuracy of PRS-RSRP measurements are tested by using the parameters in A.6.9.2.1.2-2. In all test cases, Cell 1 is the PCell.

Table A.6.9.2.1.2-1: PRS-RSRP supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table A.6.9.2.1.2-2: PRS-RSRP test parameters

| Parameter |  |  | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 |  | Cell 2 | Cell 1 | Cell 2 |  |
| Cell ID |  |  |  | 489 |  | 0 | 489 | 0 |  |
| SSB ARFCN |  |  |  | freq1 |  |  | freq1 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW |  | Config 1 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| DRX Cycle |  |  | ms | 1280 |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  | - | SR.1.1 FDD | - |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  | SR2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  | - | CR.1.1 FDD | - |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  | CR2.1 TDD |  |  |
| Control channel RMC |  | Config 1 |  | CCR.1.1 FDD |  | - | CCR.1.1 FDD | - |  |
|  |  | Config 2 |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | Config 3 |  | CCR2.1 TDD |  |  | CCR2.1 TDD |  |  |
| PRS configuration |  | Config 1 |  | PRS.1.3 FR1 |  | PRS.1.3 FR1 | PRS.1.4 FR1 | PRS.1.4 FR1 |  |
|  |  | Config 2 |  | PRS.1.3 FR1 |  | PRS.1.3 FR1 | PRS.1.4 FR1 | PRS.1.4 FR1 |  |
|  |  | Config 3 |  | PRS.2.3 FR1 |  | PRS.2.3 FR1 | PRS.2.4 FR1 | PRS.2.4 FR1 |  |
| PRS Resource slot offset (slot) |  | Config 1,2,3 | slot | 0 |  | 4 | 0 | 4 |  |
| SSB configuration |  | Config 1 |  | SSB.1 FR1 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |  |
|  |  | Config 2 |  | SSB.1 FR1 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |  |
|  |  | Config 3 |  | SSB.2 FR1 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |  |
| Time offset with Cell 1 |  | Config 1 | ms | - |  | 3 | - | 3 |  |
|  |  | Config 2,3 | s | - |  | 3 | - | 3 |  |
| SMTC configuration |  | Config 1 |  | SMTC.2 |  |  |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  | 0 | 0 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | dBm/15KhZ | -106 |  |  | -88 |  |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H |  | Not applicableNote 5 |  |  | -94 |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -106 |  |  | -88 |  |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H |  | Not applicableNote 5 |  |  | -91 |  |  |
|  |  |  | dB | 2.46 | -5.97 |  | 2.46 |  | -5.97 |
|  |  |  | dB | 6 | 1 |  | 6 |  | 1 |
| PRS-RSRP Note3 | Config 1, 2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | dBm/SCS | -100 |  | -105 | -82 |  | -87 |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H |  | Not applicable Note 5 |  | Not applicable Note 5 | -85 |  | -90 |
| IoNote3 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | dBm/9.36MHz | -70.09 |  |  | -52.09 |  |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | dBm/38.16MHz | Not applicable Note 5 |  |  | -51.99 |  |  |
| Propagation condition |  |  |  | AWGN |  |  |  |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Subtest 1 is not used when testing with 30kHz SSB SCS. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |  |  |  |  |  |

##### A.6.9.2.1.3	Test Requirements

In each test, the absolute PRS-RSRP measurement for each cell shall fulfil the absolute accuracy requirement in clause 10.1.24.2.1. The relative PRS-RSRP measurement between the two PRS resources within the same cell shall fulfil the relative accuracy requirement in clause 10.1.24.2.2.

#### A.6.9.2.2	SA: measurement accuracy with PRS in FR1 with reduced number of samples in RRC_INACTIVE state

##### A.6.9.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the PRS-RSRP measurement accuracy is within the specified limits provided that PRS is transmitted within the initial BWP of the UE. UE can support supportedDL-PRS-ProcessingSamples-INACTIVE, and the LMF indicates the UE to perform positioning measurements with reduced number of samples NsampleNsample via reducedDL-PRS-ProcessingSamples. This test will verify the requirements in clauses 10.1.24.2.1 and 10.1.24.2.2.

##### A.6.9.2.2.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Supported test configurations are shown in table A.6.9.2.2.2-1. Both absolute and relative accuracy of PRS-RSRP measurements are tested by using the parameters in A.6.9.2.2.2-2. In all test cases, Cell 1 is the PCell.

Table A.6.9.2.2.2-1: PRS-RSRP supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table A.6.9.2.2.2-2: PRS-RSRP test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Cell ID |  |  | 489 | 0 | 489 | 0 |
| SSB ARFCN |  |  | freq1 |  | freq1 |  |
| Duplex mode | Config 1 |  | FDD |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |
| TDD configuration | Config 1 |  | Not Applicable |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |
| BWchannel | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  | Config 2 |  | 10: NRB,c = 52 |  |  |  |
|  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| BWP BW | Config 1 |  | 10: NRB,c = 52 |  |  |  |
|  | Config 2 |  | 10: NRB,c = 52 |  |  |  |
|  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| Downlink initial BWP configuration |  |  | DLBWP.0.1 |  |  |  |
| Uplink initial BWP configuration |  |  | ULBWP.0.1 |  |  |  |
| DRX Cycle |  | s | 1.28 |  |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR2.1 TDD |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel | Config 1 |  | CR.1.1 FDD | - | CR.1.1 FDD | - |
|  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR2.1 TDD |  | CR2.1 TDD |  |
| Control channel RMC | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  | Config 3 |  | CCR2.1 TDD |  | CCR2.1 TDD |  |
| PRS configuration | Config 1 |  | PRS.1.1 FR1 | PRS.1.1 FR1 | PRS.1.1 FR1 | PRS.1.1 FR1 |
|  | Config 2 |  | PRS.1.1 FR1 | PRS.1.1 FR1 | PRS.1.1 FR1 | PRS.1.1 FR1 |
|  | Config 3 |  | PRS.2.1 FR1 | PRS.2.1 FR1 | PRS.2.1 FR1 | PRS.2.1 FR1 |
| PRS BW | Config 1 |  | 48 PRBs | 48 PRBs | 48 PRBs | 48 PRBs |
|  | Config 2 |  | 48 PRBs | 48 PRBs | 48 PRBs | 48 PRBs |
|  | Config 3 |  | 48 PRBs | 48 PRBs | 48 PRBs | 48 PRBs |
| PRS Resource slot offset (slot) | Config 1,2,3 | slot | 0 | 4 | 0 | 4 |
| SSB configuration | Config 1 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  | Config 2 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  | Config 3 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |
| Time offset with Cell 1 | Config 1 | ms | - | 3 | - | 3 |
|  | Config 2,3 | s | - | 3 | - | 3 |
| SMTC configuration | Config 1 |  | SMTC.2 |  |  |  |
|  | Config 2,3 |  | SMTC.1 |  |  |  |
| OCNG Patterns |  |  | OCNG pattern 1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2 | kHz | 15 kHz |  |  |  |
|  | Config 3 |  | 30 kHz |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |
| Note2 | Config 1,2 | dBm/15Khz | -98 |  | -98 |  |
|  | Config 3 |  |  |  |  |  |
| Note2 | Config 1,2 | dBm/SCS | -98 |  | -98 |  |
|  | Config 3 |  | -95 |  | -95 |  |
|  |  | dB | -3 | -6 | -3 | -6 |
|  |  | dB | -1.44 | -3.65 | -1.44 | -3.65 |
| PRS-RSRP Note3 | Config 1, 2 | dBm/SCS | -99.44 | -101.65 | -99.44 | -101.65 |
|  | Config 3 |  | -96.44 | -98.65 | -96.44 | -98.65 |
| IoNote3 | Config 1,2 | dBm/9.36MHz | -66.73 |  | -66.73 |  |
|  | Config 3 | dBm/38.16MHz | -60.62 |  | -60.62 |  |
| Propagation condition |  |  | AWGN |  |  |  |
| Antenna configuration |  |  | 1x2 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |

##### A.6.9.2.2.3	Test Requirements

In each test, the absolute PRS-RSRP measurement for each cell shall fulfil the absolute accuracy requirement in clause 10.1.24.2.1. The relative PRS-RSRP measurement between the two PRS resources within the same cell shall fulfil the relative accuracy requirement in clause 10.1.24.2.2.

### A.6.9.3	UE Rx-Tx time difference measurements

#### A.6.9.3.1.1	UE Rx-Tx time difference measurement accuracy in FR1 SA

##### A.6.9.3.1.1.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx time difference measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.25.2. The test is conducted in AWGN propagation condition in FR1 in standalone scenario.

The supported test configurations in listed in Table A.6.9.3.1.1.1-1.

Table A.6.9.3.1.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). All cells are on the same RF channel in FR1.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE before the start of the test.

The UE is configured to transmit SRS on Cell 1 during the test.

The test equipment measures the transmit timing of the UE using the transmitted SRS and measures the receive timing using the PRS. The test equipment then compares the difference of these two timings to the UE Rx-Tx measurement reported by the UE for each cell.

##### A.6.9.3.1.1.2 Test parameters

The UE Rx-Tx time difference accuracy test parameters are given in Table A.6.9.3.1.1.1-1.

Table A.6.9.3.1.1.1-1: UE Rx-Tx time difference measurement accuracy test parameters

| Parameter | Unit | Test configuration | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| RF Channel Number |  | 1,2,3 | 1 | 1 | 1 | 1 |
| DRX | s | 1,2,3 | 1.28 |  | 1.28 |  |
| Time offset with Cell 1 | s | 1, 2, 3 | N/A | 3 | N/A | 3 |
| TDD configuration |  | 1 | N/A | N/A | N/A | N/A |
|  |  | 2 | TDDConf.1.1 | TDDConf.1.1 | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 | TDDConf.2.1 | TDDConf.2.1 | TDDConf.2.1 | TDDConf.2.1 |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD | N/A | SR.1.1 FDD | N/A |
|  |  | 2 | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD | N/A | CR.1.1 FDD | N/A |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD | N/A | CCR.1.1 FDD | N/A |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 | OP.1 | OP.1 | OP.1 |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 | N/A | DLBWP.0.1 ULBWP.0.1 | N/A |
| PRS configuration |  | 1 | PRS.1.1 FR1 | PRS.1.1 FR1 | PRS.1.1 FR1 | PRS.1.1 FR1 |
|  |  | 2 | PRS.1.1 FR1 | PRS.1.1 FR1 | PRS.1.1 FR1 | PRS.1.1 FR1 |
|  |  | 3 | PRS.2.1 FR1 | PRS.2.1 FR1 | PRS.2.1 FR1 | PRS.2.1 FR1 |
| PRS Resource slot offset | slot | 1, 2, 3 | 0 | 4 | 0 | 4 |
| SRS configuration |  | 1 | POS-SRS.1 | N/A | POS-SRS.1 | N/A |
|  |  | 2 | POS-SRS.1 | N/A | POS-SRS.1 | N/A |
|  |  | 3 | POS-SRS.2 | N/A | POS-SRS.2 | N/A |
| Note 2 | dBm/SCS | 1 | -98 |  | -98 |  |
|  |  | 2 | -98 |  | -98 |  |
|  |  | 3 | -95 |  | -95 |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  | -98 |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -2.41 | -12.12 | -2.41 | -12.12 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -2 | -10 | -2 | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -100 | -108 | -100 | -108 |
|  |  | 2 | -100 | -108 | -100 | -108 |
|  |  | 3 | -97 | -105 | -97 | -105 |
| Io | dBm/9.36 MHz | 1 | -67.67 | -67.67 | -67.67 | -67.67 |
|  | dBm/9.36 MHz | 2 | -67.67 | -67.67 | -67.67 | -67.67 |
|  | dBm/38.16 MHz | 3 | -61.57 | -61.57 | -61.57 | -61.57 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  | AWGN |  |
| Note 1: Void. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.9.3.1.1.3	Test requirements

The UE Rx-Tx time difference measurement time fulfils the UE Rx-Tx measurement accuracy requirements specified in clause 10.1.25.2 for both Cell 1 and Cell 2.

#### A.6.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples

##### A.6.9.3.2.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx time difference measurement accuracy with reduced number of samples in RRC_INACTIVE state is within the specified limits. This test will verify the requirements in clause 10.1.25.2. The test is conducted in AWGN propagation condition in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations in listed in Table A.6.9.3.2.1-1.

Table A.6.9.3.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). Both cells are on the same RF channel in FR1.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE before the start of the test. The UE is configured to measure UE Rx-Tx time difference using reduced number of samples via requestedDL-PRS-ProcessingSamples in NR-Multi-RTT-RequestLocationInformation.

UE shall be configured to enter into RRC_INACTIVE state before the start of the test.

The UE is configured to transmit positioning SRS on Cell 1 during the test.

The test equipment measures the transmit timing of the UE using the transmitted SRS and measures the receive timing using the PRS. The test equipment then compares the difference of these two timings to the UE Rx-Tx measurement reported by the UE for each cell.

##### A.6.9.3.2.2	Test parameters

The UE Rx-Tx time difference accuracy test parameters are given in Table A.6.9.3.2.2-1.

Table A.6.9.3.2.2-1: UE Rx-Tx time difference measurement accuracy test parameters

| Parameter | Unit | Test configuration | Test 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |
| RF Channel Number |  | 1,2,3 | 1 | 1 |
| DRX cycle |  | 1,2,3 | 1.28s |  |
| Time offset with Cell 1 | s | 1, 2, 3 | N/A | 3 |
| TDD configuration |  | 1 | N/A | N/A |
|  |  | 2 | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 | TDDConf.2.1 | TDDConf.2.1 |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD | N/A |
|  |  | 2 | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD | N/A |
|  |  | 2 | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD | N/A |
|  |  | 2 | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 | OP.1 |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD | N/A |
|  |  | 2 | TRS.1.1 TDD |  |
|  |  | 3 | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 | N/A |
| PRS configuration |  | 1 | PRS.1.1 FR1 | PRS.1.1 FR1 |
|  |  | 2 | PRS.1.1 FR1 | PRS.1.1 FR1 |
|  |  | 3 | PRS.2.1 FR1 | PRS.2.1 FR1 |
| PRS BW |  | 1 | 48 PRBs | 48 PRBs |
|  |  | 2 | 48 PRBs | 48 PRBs |
|  |  | 3 | 48 PRBs | 48 PRBs |
| PRS Resource slot offset | slot | 1, 2, 3 | 0 | 4 |
| SRS configuration |  | 1 | POS-SRS.1 | N/A |
|  |  | 2 | POS-SRS.1 | N/A |
|  |  | 3 | POS-SRS.2 | N/A |
| Note 2 | dBm/SCS | 1 | -98 |  |
|  |  | 2 | -98 |  |
|  |  | 3 | -95 |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRS | dB | 1 | -1.76 | -6.01 |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRS | dB | 1 | 0 | -3 |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRP Note 3 | dBm/SCS kHz | 1 | -98 | -101 |
|  |  | 2 | -98 | -101 |
|  |  | 3 | -95 | -98 |
| Io | dBm/9.36 MHz | 1 | -66.07 | -66.07 |
|  | dBm/9.36 MHz | 2 | -66.07 | -66.07 |
|  | dBm/38.16 MHz | 3 | -59.97 | -59.97 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.6.9.3.2.3	Test requirements

The UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE state fulfils the UE Rx-Tx measurement accuracy requirements specified in clause 10.1.25.2 for both Cell 1 and Cell 2.

### A.6.9.4	PRS-RSRPP measurements

#### A.6.9.4.1	SA: PRS-RSRPP measurement accuracy in FR1 in RRC INACTIVE

##### A.6.9.4.1.1	Test Purpose and Environment

The purpose of this test is to verify that the PRS-RSRPP measurement accuracy in FR1 in RRC_INACTIVE state is within the specified limits. This test will verify the requirements in clauses [10.1.24.Z.1] and [10.1.24.Z.2].

##### A.6.9.4.1.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Supported test configurations are shown in table A.6.9.4.1.2-1. Both absolute and relative accuracy of PRS-RSRPP measurements are tested by using the parameters in A.6.9.4.1.2-2. In all test cases, Cell 1 is the PCell.

Table A.6.9.4.1.2-1: PRS-RSRPP supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table A.6.9.4.1.2-2: PRS-RSRPP test parameters

| Parameter |  |  | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 |  | Cell 2 | Cell 1 | Cell 2 |  |
| Cell ID |  |  |  | 489 |  | 0 | 489 | 0 |  |
| SSB ARFCN |  |  |  | freq1 |  |  | freq1 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW |  | Config 1 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  | NA | TRS.1.1 FDD | NA |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  | NA | TRS.1.1 TDD | NA |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  | NA | TRS.1.2 TDD | NA |  |
| DRX cycle length |  | Config 1,2,3 | ms | 1280 |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  | - | SR.1.1 FDD | - |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  | SR2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  | - | CR.1.1 FDD | - |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  | CR2.1 TDD |  |  |
| Control channel RMC |  | Config 1 |  | CCR.1.1 FDD |  | - | CCR.1.1 FDD | - |  |
|  |  | Config 2 |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | Config 3 |  | CCR2.1 TDD |  |  | CCR2.1 TDD |  |  |
| PRS configuration |  | Config 1 |  | PRS.1.3 FR1 |  | PRS.1.3 FR1 | PRS.1.4 FR1 | PRS.1.4 FR1 |  |
|  |  | Config 2 |  | PRS.1.3 FR1 |  | PRS.1.3 FR1 | PRS.1.4 FR1 | PRS.1.4 FR1 |  |
|  |  | Config 3 |  | PRS.2.3 FR1 |  | PRS.2.3 FR1 | PRS.2.4 FR1 | PRS.2.4 FR1 |  |
| PRS Resource slot offset (slot) |  | Config 1,2,3 | slot | 0 |  | 4 | 0 | 4 |  |
| SSB configuration |  | Config 1 |  | SSB.1 FR1 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |  |
|  |  | Config 2 |  | SSB.1 FR1 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |  |
|  |  | Config 3 |  | SSB.2 FR1 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |  |
| Time offset with Cell 1 |  | Config 1 | ms | - |  | 3 | - | 3 |  |
|  |  | Config 2,3 | s | - |  | 3 | - | 3 |  |
| SMTC configuration |  | Config 1 |  | SMTC.2 |  |  |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  | 0 | 0 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | dBm/15KhZ | -106 |  |  | -88 |  |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H |  | Not applicableNote 5 |  |  | -94 |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -106 |  |  | -88 |  |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H |  | Not applicableNote 5 |  |  | -91 |  |  |
|  |  |  | dB | 2.46 | -5.97 |  | 2.46 |  | -5.97 |
|  |  |  | dB | 6 | 1 |  | 6 |  | 1 |
| PRS-RSRP Note3 | Config 1, 2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | dBm/SCS | -100 |  | -105 | -82 |  | -87 |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H |  | Not applicable Note 5 |  | Not applicable Note 5 | -85 |  | -90 |
| IoNote3 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | dBm/9.36MHz | -70.09 |  |  | -52.09 |  |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | dBm/38.16MHz | Not applicable Note 5 |  |  | -51.99 |  |  |
| Propagation condition |  |  |  | [Two-tap channel] Note 7 |  |  |  |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Subtest 1 is not used when testing with 30kHz SSB SCS. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification Note 7: The two-tap channel model is defined in 38.101-4 Annex B.2.4 (a = 1, τd=0.45 µs and fD=5 Hz). |  |  |  |  |  |  |  |  |  |

##### A.6.9.4.1.3	Test Requirements

In each test, the absolute PRS-RSRPP measurement for each cell shall fulfil the absolute accuracy requirement in clause [10.1.24.Z.1]. The relative PRS-RSRPP measurement between the two PRS resources within the same cell shall fulfil the relative accuracy requirement in clause [10.1.24.Z.2].

#### A.6.9.4.2	SA: measurement accuracy with reduced PRS samples in FR1 in RRC INACTIVE

##### A.6.9.4.2.1	Test Purpose and Environment

The purpose of this test is to verify that the PRS-RSRPP measurement accuracy with Nsample = 1 in FR1 in RRC_INACTIVE state is within the specified limits. This test will verify the requirements in clauses [10.1.24.Z.1] and [10.1.24.Z.2].The UE under test should support [supportedDL-PRS-ProcessingSamples-RRC-Inactive ], and the TE indicates the UE to perform positioning measurements with reduced number of samples. The PRS bandwidth is contained within the initial DL BWP and the power difference between the serving cell SS-RSRP and neighbour cell PRS-RSRP is within [6]dB, so that Nsample= 1 is assumed.

##### A.6.9.4.2.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Supported test configurations are shown in table A.6.9.4.2.2-1. Both absolute and relative accuracy of PRS-RSRPP measurements are tested by using the parameters in A.6.9.4.2.2-2. In all test cases, Cell 1 is the PCell.

Table A.6.9.4.2.2-1: PRS-RSRPP supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table A.6.9.4.2.2-2: PRS-RSRPP test parameters

| Parameter |  |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |  |
| Cell ID |  |  |  | 489 | 0 |  |
| SSB ARFCN |  |  |  | freq1 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |
|  |  | Config 2,3 |  | TDD |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |
| BWP BW |  | Config 1 |  | 10: NRB,c = 52 |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  | NA |
|  |  | Config 2 |  | TRS.1.1 TDD |  | NA |
|  |  | Config 3 |  | TRS.1.2 TDD |  | NA |
| DRX Cycle length |  | Config 1,2,3 | ms | 1280 |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  | - |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  | - |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |
| Control channel RMC |  | Config 1 |  | CCR.1.1 FDD |  | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  |  |
|  |  | Config 3 |  | CCR2.1 TDD |  |  |
| PRS configuration |  | Config 1 |  | PRS.1.4 FR1 |  | PRS.1.4 FR1 |
|  |  | Config 2 |  | PRS.1.4 FR1 |  | PRS.1.4 FR1 |
|  |  | Config 3 |  | PRS.2.4 FR1 |  | PRS.2.4 FR1 |
| PRS bandwidth |  | Config 1,2,3 |  | 48 PRBs |  | 48 PRBs |
| PRS Resource slot offset (slot) |  | Config 1,2,3 | slot | 0 |  | 4 |
| SSB configuration |  | Config 1 |  | SSB.1 FR1 |  | SSB.1 FR1 |
|  |  | Config 2 |  | SSB.1 FR1 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.2 FR1 |  | SSB.2 FR1 |
| Time offset with Cell 1 |  | Config 1 | ms | - |  | 3 |
|  |  | Config 2,3 | s | - |  | 3 |
| SMTC configuration |  | Config 1 |  | SMTC.2 |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/15KhZ | -98 |  |  |
|  | Config 3 |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |
|  | Config 3 |  |  | -95 |  |  |
|  |  |  | dB | 2.46 |  | -5.97 |
|  |  |  | dB | 6 |  | 1 |
| PRS-RSRP Note3 | Config 1, 2 |  | dBm/SCS | -92 |  | -97 |
|  | Config 3 |  |  | -89 |  | -94 |
| SS-RSRP Note3 | Config 1, 2 |  | dBm/SCS | -92 |  | -97 |
|  | Config 3 |  |  | -89 |  | -94 |
| IoNote3 | Config 1,2 |  | dBm/9.36MHz | -70.09 |  |  |
|  | Config 3 |  | dBm/38.16MHz | -63.99 |  |  |
| Propagation condition |  |  |  | [Two-tap channel] Note5 |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRS-RSRP, SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The two-tap channel model is defined in 38.101-4 Annex B.2.4 (a = 1, τd=0.45 µs and fD=5 Hz). |  |  |  |  |  |  |

##### A.6.9.4.2.3	Test Requirements

In each test, the absolute PRS-RSRPP measurement for each cell shall fulfil the absolute accuracy requirement in clause [10.1.24.Z.1]. The relative PRS-RSRPP measurement between the two PRS resources within the same cell shall fulfil the relative accuracy requirement in clause [10.1.24.Z.2].