| 3GPP TS 38.151 V0.1.0 (2020-11) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements for NR UEs (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	General	7

4.1	Relationship between minimum requirements and test requirements	7

4.2	Applicability of minimum requirements	7

5	Frequency bands	7

5.1	General	7

5.2	Operating bands	7

6	FR1 MIMO OTA requirements	7

6.1	General	7

6.2	Minimum requirement	7

7	FR2 MIMO OTA requirements	8

7.1	General	8

7.2	Minimum requirement	8

Annex A (normative): <FR1 Test methodology>	9

Annex B (normative): <FR2 Test methodology>	10

Annex C (normative): <FR1 Channel models and Validation procedure>	11

Annex D (normative): <FR2 Channel models and Validation procedure >	12

Annex E (normative): <gNB configurations>	13

Annex F (normative): <Environmental requirements>	14

Annex G (informative): Change history	15

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document establishes the Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements for NR UEs operating on frequency Range 1 and frequency rang 2, for NR standalone (SA) and NR non-standalone (NSA) operation mode.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 38.827: “Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)”.

[3]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone"

[4]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone"

[5]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios"

[6]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception"

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

DML	Data Mode Landscape

DMP	Data Mode Portrait

DMSU	Data Mode Screen Up

EUT	Equipment Under Test

FR1	Frequency Range 1

FR2	Frequency Range 2

FS	Free Space

MIMO	Multiple Input Multiple Output

MPAC	Multi-Probe Anechoic Chamber

NR	New Radio

NSA	Non-Standalone, a mode of operation where operation of an other radio is assisted with an other radio

OTA	Over The Air

PDP	Power Delay Profile

PSP	PAS Similarity Percentage

UE	User Equipment

UMa	Urban Macro

UMi	Urban Micro

4	General

4.1	Relationship between minimum requirements and test requirements

The Minimum Requirements given in this specification make no allowance for measurement uncertainty. The test specification in RAN5 will define test tolerances for FR1 and FR2 MIMO OTA. The test tolerances are used to relax the minimum requirements in this specification to create test requirements.

4.2	Applicability of minimum requirements

<Editor’s note: Detailed structure of the subclause is TBD. >

5	Frequency bands

5.1	General

NR MIMO OTA Requirements are defined separately for different frequency ranges (FR). The frequency ranges in which NR can operate according to this version of the specification are identified as described in Table 5.1-1.

Table 5.1-1: Definition of frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1 | 410 MHz – 7125 MHz |
| FR2 | 24250 MHz – 52600 MHz |

The present specification covers both FR1 and FR2 operating bands.

5.2	Operating bands

NR is designed to operate in FR1 operating bands defined in TS 38.101-1 [3] and FR2 operating bands defined in TS 38.101-2 [4]. NSA band combinations are defined in TS 38.101-3 [5]. E-UTRA is designed to operate in operating bands defined in TS 36.101 [6].

6	FR1 MIMO OTA requirements

6.1	General

<Editor’s note: Detailed structure of the subclause is TBD. Include FoM of TRMS for FR1. >

6.2	Minimum requirement

<Editor’s note: Detailed structure of the subclause is TBD. Subclause for SA and EN-DC bands can be added>

7	FR2 MIMO OTA requirements

7.1	General

<Editor’s note: Detailed structure of the subclause is TBD. Include FoM of and data processing for FR2. >

7.2	Minimum requirement

<Editor’s note: Detailed structure of the subclause is TBD. >

Annex A (normative):
<FR1 Test methodology>

<Editor’s note: Detailed structure of the subclause is TBD.

Refer to TR38.827 Include: General (environmental condition, test zone size), measurement methodology (setup, procedure, minimum range length), EUT positioning:

Exception or additional decisions can be added. >

A.1	General

FR1 MIMO OTA requirement testing is based on UE-noise limited environmental condition, i.e., UE throughput characterized as a function of signal power incident to the DUT antennas.

The minimum test zone size for FR1 MIMO OTA test methods is 20cm. “Black-box” testing approach is adopted for NR MIMO OTA testing, the physical center of the EUT shall be placed in the centre of the test zone, the EUT shall be completely contained within the minimum test zone size.

A.2	Multi-Probe Anechoic Chamber (MPAC)

A.2.1	system setup

MPAC test method is the reference methodology for FR NR MIMO OTA testing. By arranging an array of antennas around the Equipment Under Test (EUT), a spatial distribution of angles of arrival in MPAC system may be simulated to expose the EUT to a near field environment that appears to have originated from a complex multipath far field environment.

As illustrated schematically in Figure A.2.1-1, signals propagate from the base station/communication tester to the EUT through a simulated multipath environment known as a spatial channel model, where appropriate channel impairments such as Doppler and fading are applied to each path prior to injecting all of the directional signals into the chamber simultaneously through the antenna array. The resulting field distribution in the test zone is then integrated by the EUT antenna(s) and processed by the receiver(s) just as it would do so in any non-simulated multipath environment. MPAC system with 16 uniformly-spaced dual-polarized probes is permitted for NR FR1 MIMO OTA testing.

Figure A.2.1-1: MPAC system layout for NR FR1 MIMO OTA testing

A.2.2	Calibration procedure

The system needs to be calibrated by using a reference calibration antenna with known gain values in order to ensure that the downlink signal power is correct. In non-standalone (NSA) mode, the LTE link antenna provides a stable LTE signal without precise path loss or polarization control.

Unlike traditional TRP/TRS testing where the path loss corrections can all be applied as a post processing step to the measured data, the path loss for each probe in the MPAC system must be balanced at test time in order to generate the desired channel model environment within the test zone of the chamber. The imbalance of each path during testing would result in an alteration of the angular dependence of the channel model (i.e. varied characteristics of generated channel model) within the test zone of the chamber.

1. Place a vertical reference dipole in the center of the test zone, connected to a VNA port, with the other VNA port connected to the input of the channel emulator unit.

2. Configure the channel emulator for bypass mode.

3. Measure the response of each path from each vertical polarization probe to the reference antenna in the center of test zone.

4. Adjust the power on all vertical polarization branches of the channel emulator so that the powers received at the center are equal.

5. Repeat the steps 1 to 4 with the magnetic loop and horizontally polarized probes instead, and adjust the horizontal polarization branches of the channel emulator.

6. The worst-case path loss becomes the reference path loss of the entire system, this loss is used to compute the power in the center of the test zone relative to the output power of the Base Station simulator. Besides, based on the reference path loss, the relative offset of each path loss shall be corrected.

Note: calibration based on other antennas, e.g., horn antennas is not precluded.

A.2.3	Test procedure

Before throughput testing, the initial conditions shall be confirmed to reach the correct measurement state for each test case.

1. Ensure environmental requirements of Annex F are met.

2. Configure the test system according to Annex C, D and E for the applicable test case.

3. Verify the implementation of the channel model as specified in Annex C.3.

4. Position the UE in the chamber according to Annex A.3.

5. Power on the UE.

6. Set up the connection.

Note:        For step 3, the verification of the channel model implementation is usually performed once for each channel model as part of the laboratory accreditation process, and will remain valid as long as the setup and instruments remain unchanged. Otherwise the channel model validation may need to be performed prior to starting each throughput test.

For throughput testing, the following steps shall be followed in order to evaluate NR MIMO OTA performance of the DUT:

1. Measure MIMO OTA throughput from one measurement point, the maximum downlink power is TBD. MIMO OTA throughput is the minimum downlink signal power resulting in a pre-defined throughput value, i.e., 70% of the maximum theoretical throughput.  The downlink signal power step size shall be no more than 0.5 dB when RF power level is near the NR MIMO sensitivity level.

2. Rotate the UE around vertical axis of the test system by 30 degrees and repeat from step 1 until one complete rotation has been measured i.e. 12 different UE azimuth rotations.

3. Repeat the test from step 1 for each specified device orientation. A list of orientations is given in Annex A.3.

4. The postprocessing method to calculate the average MIMO Throughput is defined in Clause 6.

A.2.4	Minimum Range Length

The minimum range length of FR1 MPAC system is defined as the distance from the centre of the test zone to the aperture of the measurement probes/antennas, as illustrated in Figure A.2.4-1.

Figure A.2.4-1: Illustration of range length definition of FR1 MPAC

The minimum range length for NR FR1 MPAC OTA systems with 20cm test zone size is 1.2m. While for MPAC systems, the far-field requirements do not have to apply, it was shown that the spatial correlation can be impacted significantly for distances below 1.2m.

A.3	EUT positioning

This Clause defines the measurement coordinate system for the NR MIMO OTA.

For FR1 MIMO OTA, the DUT shall be tested under Free Space Data Mode Portrait (FS DMP), Free Space Data Mode Landscape (FS DML), and Free Space Data Mode Screen Up flat (FS DMSU), the DUT azimuthal rotation shall be performed over 360 degrees per orientation in 30 degree steps (12 total positions).

Annex B (normative):
<FR2 Test methodology>

<Editor’s note: Detailed structure of the subclause is TBD.

Refer to TR38.827 Include: General (environmental condition, test zone size), measurement methodology (setup, procedure, minimum range length), EUT positioning:

Exception or additional decisions can be added. >

Annex C (normative):
<FR1 Channel models and Validation procedure>

<Editor’s note: Detailed structure of the subclause is TBD.

Include FR1 channel model tables after down-selection for requirement, FR1 channel model validation procedure, and validation pass/fail requirement. >

C.1	FR1 Channel models

The following channel models are required for FR1 MIMO OTA measurement.

The generic models are Table C.1-1 FR1 UMi CDL-A and Table C.1-2 FR1 UMa CDL-C, which do not include base station antenna filtering.

Therefore, in addition, the BS beam filtering effect defined in Annex C.2 also apply when emulating the channel models.

Table C.1-1: Channel model parameters for UMi CDL-A at 3.5 GHz

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -13.4014 | -59.324 | 98.721 | 95.9936 | 90 |
| 2 | 38.19 | 0 | -2.752 | -156.546 | 97.1624 | 90 |
| 3 | 40.25 | -2.2185 | -2.752 | -156.546 | 97.1624 | 90 |
| 4 | 58.68 | -3.9794 | -2.752 | -156.546 | 97.1624 | 90 |
| 5 | 46.1 | -5.9799 | 27.9576 | 115.7066 | 97.9452 | 90 |
| 6 | 53.75 | -8.1984 | 27.9576 | 115.7066 | 97.9452 | 90 |
| 7 | 67.08 | -9.9593 | 27.9576 | 115.7066 | 97.9452 | 90 |
| 8 | 57.5 | -10.5014 | 38.1399 | -55.2369 | 98.7118 | 90 |
| 9 | 76.18 | -7.5014 | -27.9638 | -82.1587 | 96.1295 | 90 |
| 10 | 153.75 | -15.9014 | 50.144 | 127.5226 | 95.3467 | 90 |
| 11 | 189.78 | -6.6014 | -28.3867 | 99.1238 | 98.0648 | 90 |
| 12 | 222.42 | -16.7014 | 42.4666 | -131.84 | 99.2935 | 90 |
| 13 | 217.18 | -12.4014 | -51.1586 | 82.1383 | 98.7444 | 90 |
| 14 | 249.42 | -15.2014 | -57.3396 | 115.7066 | 98.902 | 90 |
| 15 | 251.19 | -10.8014 | -43.6439 | -58.728 | 95.912 | 90 |
| 16 | 305.82 | -11.3014 | -45.6283 | -69.4699 | 95.7272 | 90 |
| 17 | 408.1 | -12.7014 | 52.4212 | -85.7169 | 95.806 | 90 |
| 18 | 445.79 | -16.2014 | 46.8909 | 138.3987 | 99.0271 | 90 |
| 19 | 456.95 | -18.3014 | 41.7835 | 161.2923 | 94.9226 | 90 |
| 20 | 479.66 | -18.9014 | -39.9679 | 168.543 | 95.083 | 90 |
| 21 | 500.66 | -16.6014 | -51.5165 | 132.7593 | 99.2962 | 90 |
| 22 | 530.43 | -19.9014 | 39.7665 | -155.942 | 95.2461 | 90 |
| 23 | 965.86 | -29.7014 | -19.6683 | 101.3393 | 98.5677 | 90 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 1.6266 | 7.385 | 0.0815 | 0 | 10 |  |

Table C.1-2: Channel model parameters for UMa CDL-C at 3.5 GHz

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4215 | -37.4195 | -96.4031 | 96.7645 | 90 |
| 2 | 76.6135 | -1.25 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 3 | 80.9935 | -3.4684 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 4 | 85.0085 | -5.2294 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 5 | 79.424 | -2.5215 | -33.5316 | -124.0196 | 100.8594 | 90 |
| 6 | 232.359 | 0 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 7 | 235.352 | -2.2185 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 8 | 239.44 | -3.9794 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 9 | 240.316 | -7.4215 | 41.4581 | 51.4188 | 106.3995 | 90 |
| 10 | 289.6275 | -7.1215 | -49.2149 | 62.9864 | 94.4761 | 90 |
| 11 | 299.7745 | -10.7215 | 46.1367 | -41.2744 | 107.4834 | 90 |
| 12 | 340.764 | -11.1215 | -70.697 | 42.5606 | 92.3083 | 90 |
| 13 | 448.4025 | -5.1215 | -43.1524 | 64.6538 | 104.5929 | 90 |
| 14 | 477.5295 | -6.8215 | -49.0831 | -62.7423 | 105.1951 | 90 |
| 15 | 792.196 | -8.7215 | -58.4403 | 78.6184 | 91.7061 | 90 |
| 16 | 989.3325 | -13.2215 | 60.9633 | 25.6781 | 105.1951 | 90 |
| 17 | 1554.4985 | -13.9215 | 58.6569 | -23.4063 | 93.9944 | 90 |
| 18 | 1679.1095 | -13.9215 | 51.8037 | -2.3553 | 91.8265 | 90 |
| 19 | 2003.923 | -15.8215 | -73.86 | -20.5926 | 90.7426 | 90 |
| 20 | 2046.8105 | -17.1215 | 54.0442 | 3.7933 | 108.2061 | 90 |
| 21 | 2301.8725 | -16.0215 | 54.7691 | -0.4794 | 91.7061 | 90 |
| 22 | 2422.651 | -15.7215 | 63.5332 | -5.5859 | 91.5856 | 90 |
| 23 | 2570.5855 | -21.6215 | 72.0338 | -29.1381 | 106.3995 | 90 |
| 24 | 3158.0895 | -22.8215 | -88.2912 | 28.7003 | 109.5309 | 90 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 1.3179 | 15.632 | 3.6131 | 0 | 7 |  |

C.2	FR1 Base Station beam configuration

The emulated BS beam configuration to be used for all emulation of channel models defined in Annex C.1 is specified in this clause.

The Base Station beam configuration includes basic antenna parameters and beamforming characteristic. The basic BS antenna parameters is defined in Table C.2-1.

Table C.2-1: BS Antenna Parameters

| Parameter description | Symbol | Parameter value |  |  |
| --- | --- | --- | --- | --- |
|  |  | FR1 ≤2.5GHz | FR1 >2.5GHz | FR2 |
| Antenna panels in vertical dimension | Mg | 1 | 1 | 1 |
| Antenna panels in horizontal dimension | Ng | 1 | 1 | 1 |
| Elements per panel in vertical dimension | Me | 4 | 8 | 8 |
| Elements per panel in horizontal dimension | Ne | 8 | 8 | 16 |
| Number of polarizations per panel | P | 2 | 2 | 2 |
| Element spacing in horizontal dimension () | dH | 0.5 | 0.5 | 0.5 |
| Element spacing in vertical dimension () | dV | 0.5 | 0.5 | 0.5 |

The beamforming characteristic of the FR1 BS pattern is defined as follow:

- A code book of 60 fixed beams is constructed to a grid of five elevation angles from –20 to +20 with 10 steps and 12 azimuth angles from –80 to +80 with ~15 steps；

- 2 strongest transmitting beams are selected from the pre-defined beam grid based on their proximity to the strong clusters of each FR1 channel model.

C.3	FR1 Channel model validation

C.3.1	General

This clause describes the MIMO OTA validation measurements, in order to ensure that the channel models are correctly implemented and hence capable of generating the propagation environment, as described by the model, within the test zone.

The following measurements shall be done for FR1 channel model validation:

Power Delay Profile (PDP)

Doppler/Temporal correlation

Spatial correlation

Cross-polarization

Power validation

Frequencies to be used to test for channel model validation:

Table C.3.1-1: Channel model validation frequencies

| NR FR1 Bands | Range | Test frequency (MHz) |
| --- | --- | --- |
| n71 | Low | 617MHz |
| n12, n17, n29, n14, n28, [n29] |  | 722MHz |
| n5, n8, n18, n20 |  | 836.5MHz |
| n50, n51, n74 | Mid | 1575.42MHz |
| n3, n2, n25, n39 |  | 1880MHz |
| n1, n34, n65 |  | 2132.5MHz |
| n7, n30, n41, n40, n38, [n90] |  | 2450MHz |
| n77,n78 | High | 3600MHz |
| n79 |  | [4700MHz] |

C.3.2	Power Delay Profile (PDP)

This measurement checks that the resulting power delay profile (PDP) is in-line with the PDP defined for the channel model. For PDP validation measurement, only Vertical validation is required.

The PDP measurement is performed with a Vector Network Analyzer (VNA). An example setup for PDP measurement is shown in Figure C.3.2-1. VNA transmits frequency sweep signals thorough the NR MIMO OTA test system. A reference antenna (i.e dipole antenna), within the center of the test zone, receives the signal and VNA analyses the frequency response of the system. A number of traces (frequency responses) are measured and recorded by VNA and analyzed by a post processing SW, e.g., Matlab. Special care has to be taken into account to keep the fading conditions unchanged, i.e. frozen, during the short period of time of a single trace measurement. The fading may proceed only in between traces.

Figure C.3.2-1: Setup for PDP measurements

Step the emulation and store traces from VNA. I.e. run the emulation to CIR number 1, pause, measure VNA trace, run the emulation to CIR number 10, pause, measure VNA trace. Continue until 1000 VNA traces are measured.

VNA settings:

Table C.3.2-1: VNA settings for PDP measurements

| Item | Unit | Value |
| --- | --- | --- |
| Center frequency | MHz | Downlink center frequency in Table C.3.1-1 |
| Span | MHz | 200 |
| Number of traces |  | 1000 |
| Number of points |  | 1101 |
| Averaging |  | 1 |

Channel model specification:

Table C.3.2-2: Channel model specification for PDP measurements

| Item | Unit | Value |
| --- | --- | --- |
| Center frequency | MHz | Downlink center frequency in Table C.3.1-1 |
| Distance between traces in channel model | wavelength (Note) | > 2 |
| Channel model |  | As specified in Annex C.1 |
| NOTE: Time [s] = distance [] / MS speed [/s] MS speed [/s] = MS speed [m/s] / Speed of light [m/s] * Center frequency [Hz] |  |  |

Method of measurement result analysis:

Measured VNA traces (frequency responses H(t,f)) are saved into a hard drive. The data is read into, e.g., Matlab. 
The analysis is performed by taking the Fourier transform of each FR. The resulting impulse responses h(t,) are averaged in power over time:

Finally the resulting PDP is shifted in delay, such that the first tap is on delay zero.

C.3.3	Doppler/Temporal correlation

This measurement checks the Doppler/temporal correlation. For Doppler/Temporal correlation validation measurement, only Vertical validation is required.

The Doppler spectrum is measured with a spectrum analyzer as shown in Figure C.3.3-1. In this case a signal generator transmits CW signal through the NR MIMO OTA test system. The signal is received by a test antenna within the test area. Finally, the signal is analyzed by a spectrum analyzer and the measured spectrum is compared to the target spectrum. This setup can be used to measure Doppler Spectrum of the Channel models defined in Annex C.1.

Method of measurement:

Figure C.3.3-1: Setup for Doppler measurements

Sine wave (CW, carrier wave) signal is transmitted from the signal generator. The signal is connected from the signal generator to fading emulator via cables. The fading emulator output signals are connected to power amplifier boxes via cables. The amplified signals are then transferred via cables to the probe antennas. The probe antennas radiate the signals over the air to the test antenna The Doppler spectrum is measured by the spectrum analyzer and the trace is saved.

Signal generator settings:

Table C.3.3-1: Signal generator settings for Doppler/Temporal correlation measurements

| Item | Unit | Value |
| --- | --- | --- |
| Center frequency | MHz | Downlink center frequency in Table C.3.1-1 |
| Modulation |  | OFF |

Spectrum analyzer settings:

Table C.3.3-2: Spectrum analyzer settings for Doppler/Temporal correlation measurements

| Item | Unit | Value |
| --- | --- | --- |
| Center frequency | MHz | Downlink center frequency in Table C.3.1-1 |
| Minimum Span | Hz | 4 kHz |
| RBW | Hz | 1 |
| VBW | Hz | 1 |
| Number of points |  | 16002 |
| Averaging |  | 100 |

Channel model specification:

Table C.3.3-3: Channel model specification for Doppler/Temporal correlation measurements

| Item | Unit | Value |
| --- | --- | --- |
| Center frequency | MHz | Downlink center frequency in Table C.3.1-1 |
| Channel model |  | As specified in Annex C.1 |
| Mobile speed | km/h | 100 |

Method of measurement result analysis: Measurement data file (Doppler power spectrum) is saved into hard drive. The data is read into, e.g., Matlab. The analysis is performed by taking the Fourier transformation of the Doppler spectrum. The resulting temporal correlation function   is normalized such that . Then the function values left from the maximum is cut out. Further on the function values after, e.g. seven periods is cut out.

C.3.4	Spatial correlation

This measurement checks whether the measured correlation curve follows the theoretical curve. For spatial correlation validation measurement, both Vertical and Horizontal validation are required. Spatial correlation validation is only adopted for FR1 MIMO OTA.

The spatial correlation validation measurement setup is illustrated in Figure C.3.4-1. The network analyser transmits signals through the fading emulator and probes. The 16 probes radiate the signals within the anechoic chamber and a receiving test antenna is placed within the test zone. The test antenna is attached to a positioner that can move the antenna to pre-defined spatial locations on a fixed radius from the centre of the quiet zone. The received signal is measured with the network analyser.

The measurement and analysis procedure is as follows:

- Set the target channel model to fading emulator.

- For each position of the test antenna in the test zone, step & pause the emulator to different time instances. Measure the frequency responses  for all stepped channel snapshots , where the interval between frequency and time samples is  and , respectively. The number of channel snapshots N and frequency samples M should be sufficiently high so that the matrix can be estimated reliably.

- Move the measurement antenna with a positioner to another location k and repeat step 2 to record frequency responses  of all stepped channel snapshots.

- Repeat step 3 to record frequency responses at all  spatial sample points.

- Stack measured time and frequency samples to a vector and calculate correlation between the first spatial sample point (i.e. k=1) and other spatial points

- Take the theoretical reference spatial correlation of the corresponding spatial sample points. Plot both the measured and theoretical curves.

- Calculate the weighted RMS correlation error between the measured and the reference.

Figure C.3.4-1: Configuration for spatial correlation validation

Time and frequency samples

The number of temporal snapshots N and frequency samples M is TBD. They must be chosen to minimize the validation measurement time, but sufficiently high to keep an adequate correlation estimation accuracy. It is beneficial to choose the time sampling interval  larger than the coherence time of the channel model. such that the recorded time samples represent independent fading occasions. The same principle applies also to frequency sampling interval  and the channel coherence bandwidth.

Spatial samples

The spatial samples for the correlation validation measurement are on the circumference of the quiet zone, as illustrated in Figure C.3.4-2. The test zone is a circle with 20 cm diameter in the horizontal plane. The reference point (denoted by a red marker) is in AoA 270. The mean AoAs of the CDL-A and CDL-C models are slightly different, but the underlying geometry for the CDL model indicates that the mean AoA (or assumed LoS direction) of the model is 180°. The reference point orientation of the validation measurement is proposed to be with 90° offset to the channel model reference AoA to enable accurate sampling of the main lobe of the spatial correlation curve. The reference point orientation must be defined in the channel model coordinate system instead of the chamber/probe coordinate system to enable optimization of OTA model implementation to achieve better alignment with the cluster AoAs and probe directions. In order to have spatial samples that yield reasonable measurement times and adequately capture the main lobe of the correlation curve, a non-uniform sampling is used where the first quadrant i.e., 270-180, is sampled with dense sampling compared to the rest of the circle. The spacing of the spatial samples is summarized in Table C.3.4-1 for test frequencies less than 1800 MHz and equal to or greater than 1800 MHz.

Table C.3.4-1: Spacing of Spatial Samples

| Test Frequencies [MHz] | First quadrant of test zone circumference (270o-180o) | Remaining quadrants |
| --- | --- | --- |
| 617, 722, 836.5 1575.42 | /15 | /4 |
| 1800, 2132.50, 2450, 3600, 4700 | /10 | /2 |

Figure C.3.4-2: Spatial sampling for spatial correlation validation measurement for test frequencies less than and equal to or greater than 1800 MHz: 617 MHz spatial sampling (left) and 4700 MHz spatial sampling (right).

Reference Spatial Correlation Curves

The spatial correlation validation reference curves are tabulated in Tables C.3.4-2 and C.3.4-3 for CDL-A UMi and in Tables C.3.4-4 and C.3.4-5 for CDL-C UMa for a vertically polarized MPAC OTA setup with 16 uniformly spaced probes.

Table C.3.4-2: Spatial correlation reference curves for CDL-A UMi model for a vertically polarized MPAC OTA setup with 16 uniformly spaced probes at FR1 test frequencies below 1800 MHz

| 617 MHz |  | 722 MHz |  | 836.5 MHz |  | 1575.42 MHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Azimuth [o] | ref | Azimuth [o] | ref | Azimuth [o] | ref | Azimuth [o] | ref |
| 270.0 | 1.000 | 270.0 | 1.000 | 270.0 | 1.000 | 270.0 | 1.000 |
| 251.4 | 0.999 | 254.1 | 0.999 | 256.3 | 0.999 | 262.7 | 0.999 |
| 232.9 | 0.997 | 238.3 | 0.997 | 242.6 | 0.996 | 255.5 | 0.996 |
| 214.3 | 0.992 | 222.4 | 0.993 | 228.9 | 0.993 | 248.2 | 0.992 |
| 195.8 | 0.981 | 206.6 | 0.984 | 215.2 | 0.987 | 240.9 | 0.987 |
| 110.40 | 0.809 | 190.7 | 0.969 | 201.6 | 0.975 | 233.7 | 0.982 |
| 40.80 | 0.823 | 120.52 | 0.778 | 187.9 | 0.955 | 226.4 | 0.977 |
| 331.21 | 0.96 | 61.05 | 0.731 | 128.66 | 0.751 | 219.1 | 0.971 |
|  |  | 1.57 | 0.88 | 77.33 | 0.645 | 211.9 | 0.962 |
|  |  | 302.09 | 0.99 | 25.99 | 0.762 | 204.6 | 0.949 |
|  |  |  |  | 334.66 | 0.928 | 197.3 | 0.929 |
|  |  |  |  | 283.32 | 0.998 | 190.0 | 0.903 |
|  |  |  |  |  |  | 182.8 | 0.868 |
|  |  |  |  |  |  | 152.74 | 0.620 |
|  |  |  |  |  |  | 125.48 | 0.363 |
|  |  |  |  |  |  | 98.23 | 0.299 |
|  |  |  |  |  |  | 70.97 | 0.364 |
|  |  |  |  |  |  | 43.71 | 0.460 |
|  |  |  |  |  |  | 16.45 | 0.58 |
|  |  |  |  |  |  | 349.20 | 0.71 |
|  |  |  |  |  |  | 321.94 | 0.86 |
|  |  |  |  |  |  | 294.68 | 0.97 |

Table C.3.4-3: Spatial correlation reference curves for CDL-A UMi model for a vertically polarized MPAC OTA setup with 16 uniformly spaced probes at FR1 test frequencies equal to or greater than 1800 MHz

| 1800 MHz |  | 2132.5 MHz |  | 2450 MHz |  | 3600 MHz |  | 4700 MHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Azimuth [o] | ref | Azimuth [o] | ref | Azimuth [o] | ref | Azimuth [o] | ref | Azimuth [o] | ref |
| 270.0 | 1.000 | 270.0 | 1.000 | 270.0 | 1.000 | 270.0 | 1.000 | 270.0 | 1.000 |
| 260.9 | 0.998 | 261.9 | 0.998 | 263.0 | 0.997 | 265.2 | 0.997 | 266.3 | 0.997 |
| 251.7 | 0.991 | 253.9 | 0.990 | 256.0 | 0.990 | 260.5 | 0.990 | 262.7 | 0.990 |
| 242.6 | 0.981 | 245.8 | 0.980 | 249.0 | 0.979 | 255.7 | 0.979 | 259.0 | 0.979 |
| 233.5 | 0.967 | 237.8 | 0.967 | 242.0 | 0.966 | 250.9 | 0.966 | 255.4 | 0.969 |
| 224.3 | 0.951 | 229.7 | 0.952 | 234.9 | 0.951 | 246.1 | 0.950 | 251.7 | 0.956 |
| 215.2 | 0.932 | 221.7 | 0.933 | 227.9 | 0.932 | 241.4 | 0.932 | 248.1 | 0.942 |
| 206.0 | 0.906 | 213.6 | 0.911 | 220.9 | 0.913 | 236.6 | 0.908 | 244.4 | 0.922 |
| 196.9 | 0.877 | 205.6 | 0.883 | 213.9 | 0.888 | 231.8 | 0.881 | 240.8 | 0.896 |
| 187.8 | 0.845 | 197.5 | 0.854 | 206.9 | 0.862 | 227.1 | 0.857 | 237.1 | 0.872 |
| 134.3 | 0.748 | 189.5 | 0.823 | 199.9 | 0.833 | 222.3 | 0.832 | 233.5 | 0.842 |
| 88.6 | 0.729 | 181.4 | 0.795 | 192.9 | 0.805 | 217.5 | 0.815 | 229.8 | 0.817 |
| 43.0 | 0.833 | 139.7 | 0.737 | 185.9 | 0.783 | 212.7 | 0.800 | 226.1 | 0.792 |
| 357.3 | 0.953 | 99.5 | 0.725 | 144.9 | 0.742 | 208.0 | 0.792 | 222.5 | 0.775 |
| 311.6 | 0.978 | 59.2 | 0.753 | 109.9 | 0.754 | 203.2 | 0.785 | 218.8 | 0.760 |
|  |  | 18.9 | 0.884 | 74.8 | 0.727 | 198.4 | 0.782 | 215.2 | 0.753 |
|  |  | 338.6 | 0.970 | 39.8 | 0.778 | 193.7 | 0.781 | 211.5 | 0.750 |
|  |  | 298.4 | 0.982 | 4.7 | 0.901 | 188.9 | 0.786 | 207.9 | 0.753 |
|  |  |  |  | 329.7 | 0.974 | 184.1 | 0.795 | 204.2 | 0.760 |
|  |  |  |  | 294.6 | 0.980 | 156.1 | 0.886 | 200.6 | 0.775 |
|  |  |  |  |  |  | 132.3 | 0.952 | 196.9 | 0.792 |
|  |  |  |  |  |  | 108.4 | 0.949 | 193.3 | 0.817 |
|  |  |  |  |  |  | 84.6 | 0.906 | 189.6 | 0.840 |
|  |  |  |  |  |  | 60.7 | 0.830 | 185.9 | 0.865 |
|  |  |  |  |  |  | 36.9 | 0.741 | 182.3 | 0.888 |
|  |  |  |  |  |  | 13.0 | 0.774 | 161.7 | 0.978 |
|  |  |  |  |  |  | 349.1 | 0.894 | 143.5 | 0.945 |
|  |  |  |  |  |  | 325.3 | 0.966 | 125.2 | 0.926 |
|  |  |  |  |  |  | 301.4 | 0.969 | 106.9 | 0.926 |
|  |  |  |  |  |  | 277.6 | 0.994 | 88.6 | 0.948 |
|  |  |  |  |  |  |  |  | 70.4 | 0.948 |
|  |  |  |  |  |  |  |  | 52.1 | 0.896 |
|  |  |  |  |  |  |  |  | 33.8 | 0.747 |
|  |  |  |  |  |  |  |  | 15.5 | 0.682 |
|  |  |  |  |  |  |  |  | 357.3 | 0.799 |
|  |  |  |  |  |  |  |  | 339.0 | 0.912 |
|  |  |  |  |  |  |  |  | 320.7 | 0.956 |
|  |  |  |  |  |  |  |  | 302.4 | 0.968 |
|  |  |  |  |  |  |  |  | 284.2 | 0.973 |

Table C.3.4-4: Spatial correlation reference curves for CDL-C UMa model for a vertically polarized MPAC OTA setup with 16 uniformly spaced probes at FR1 test frequencies below 1800 MHz

| 617 MHz |  | 722 MHz |  | 836.5 MHz |  | 1575.42 MHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Azimuth [o] | ref | Azimuth [o] | ref | Azimuth [o] | ref | Azimuth [o] | ref |
| 270.0 | 1.000 | 270.0 | 1.000 | 270.0 | 1.000 | 270.0 | 1.000 |
| 251.4 | 0.999 | 254.1 | 0.999 | 256.3 | 0.999 | 262.7 | 0.999 |
| 232.9 | 0.997 | 238.3 | 0.997 | 242.6 | 0.996 | 255.5 | 0.996 |
| 214.3 | 0.992 | 222.4 | 0.993 | 228.9 | 0.993 | 248.2 | 0.992 |
| 195.8 | 0.981 | 206.6 | 0.984 | 215.2 | 0.987 | 240.9 | 0.987 |
| 110.40 | 0.809 | 190.7 | 0.969 | 201.6 | 0.975 | 233.7 | 0.982 |
| 40.80 | 0.823 | 120.52 | 0.778 | 187.9 | 0.955 | 226.4 | 0.977 |
| 331.21 | 0.96 | 61.05 | 0.731 | 128.66 | 0.751 | 219.1 | 0.971 |
|  |  | 1.57 | 0.88 | 77.33 | 0.645 | 211.9 | 0.962 |
|  |  | 302.09 | 0.99 | 25.99 | 0.762 | 204.6 | 0.949 |
|  |  |  |  | 334.66 | 0.928 | 197.3 | 0.929 |
|  |  |  |  | 283.32 | 0.998 | 190.0 | 0.903 |
|  |  |  |  |  |  | 182.8 | 0.868 |
|  |  |  |  |  |  | 152.74 | 0.620 |
|  |  |  |  |  |  | 125.48 | 0.363 |
|  |  |  |  |  |  | 98.23 | 0.299 |
|  |  |  |  |  |  | 70.97 | 0.364 |
|  |  |  |  |  |  | 43.71 | 0.460 |
|  |  |  |  |  |  | 16.45 | 0.58 |
|  |  |  |  |  |  | 349.20 | 0.71 |
|  |  |  |  |  |  | 321.94 | 0.86 |
|  |  |  |  |  |  | 294.68 | 0.97 |

Table C.3.4-5: Spatial correlation reference curves for CDL-C UMa model for a vertically polarized MPAC OTA setup with 16 uniformly spaced probes at FR1 test frequencies equal to or greater than 1800 MHz

| 1800 MHz |  | 2132.5 MHz |  | 2450 MHz |  | 3600 MHz |  | 4700 MHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Azimuth [o] | ref | Azimuth [o] | ref | Azimuth [o] | ref | Azimuth [o] | ref | Azimuth [o] | ref |
| 270.0 | 1.000 | 270.0 | 1.000 | 270.0 | 1.000 | 270.0 | 1.000 | 270.0 | 1.000 |
| 260.9 | 0.998 | 261.9 | 0.998 | 263.0 | 0.997 | 265.2 | 0.997 | 266.3 | 0.996 |
| 251.7 | 0.991 | 253.9 | 0.991 | 256.0 | 0.992 | 260.5 | 0.989 | 262.7 | 0.988 |
| 242.6 | 0.984 | 245.8 | 0.984 | 249.0 | 0.983 | 255.7 | 0.979 | 259.0 | 0.976 |
| 233.5 | 0.976 | 237.8 | 0.975 | 242.0 | 0.975 | 250.9 | 0.969 | 255.4 | 0.966 |
| 224.3 | 0.967 | 229.7 | 0.966 | 234.9 | 0.965 | 246.1 | 0.960 | 251.7 | 0.957 |
| 215.2 | 0.955 | 221.7 | 0.955 | 227.9 | 0.953 | 241.4 | 0.954 | 248.1 | 0.951 |
| 206.0 | 0.936 | 213.6 | 0.940 | 220.9 | 0.939 | 236.6 | 0.947 | 244.4 | 0.945 |
| 196.9 | 0.908 | 205.6 | 0.918 | 213.9 | 0.921 | 231.8 | 0.937 | 240.8 | 0.940 |
| 187.8 | 0.863 | 197.5 | 0.888 | 206.9 | 0.898 | 227.1 | 0.925 | 237.1 | 0.935 |
| 134.3 | 0.309 | 189.5 | 0.846 | 199.9 | 0.867 | 222.3 | 0.903 | 233.5 | 0.928 |
| 88.6 | 0.269 | 181.4 | 0.793 | 192.9 | 0.829 | 217.5 | 0.876 | 229.8 | 0.918 |
| 43.0 | 0.396 | 139.7 | 0.280 | 185.9 | 0.786 | 212.7 | 0.837 | 226.1 | 0.902 |
| 357.3 | 0.619 | 99.5 | 0.252 | 144.9 | 0.245 | 208.0 | 0.798 | 222.5 | 0.882 |
| 311.6 | 0.879 | 59.2 | 0.257 | 109.9 | 0.299 | 203.2 | 0.753 | 218.8 | 0.851 |
|  |  | 18.9 | 0.471 | 74.8 | 0.215 | 198.4 | 0.708 | 215.2 | 0.816 |
|  |  | 338.6 | 0.661 | 39.8 | 0.251 | 193.7 | 0.669 | 211.5 | 0.767 |
|  |  | 298.4 | 0.937 | 4.7 | 0.489 | 188.9 | 0.624 | 207.9 | 0.708 |
|  |  |  |  | 329.7 | 0.652 | 184.1 | 0.580 | 204.2 | 0.651 |
|  |  |  |  | 294.6 | 0.946 | 156.1 | 0.175 | 200.6 | 0.580 |
|  |  |  |  |  |  | 132.3 | 0.565 | 196.9 | 0.516 |
|  |  |  |  |  |  | 108.4 | 0.745 | 193.3 | 0.444 |
|  |  |  |  |  |  | 84.6 | 0.820 | 189.6 | 0.383 |
|  |  |  |  |  |  | 60.7 | 0.750 | 185.9 | 0.310 |
|  |  |  |  |  |  | 36.9 | 0.493 | 182.3 | 0.229 |
|  |  |  |  |  |  | 13.0 | 0.120 | 161.7 | 0.445 |
|  |  |  |  |  |  | 349.1 | 0.272 | 143.5 | 0.750 |
|  |  |  |  |  |  | 325.3 | 0.498 | 125.2 | 0.879 |
|  |  |  |  |  |  | 301.4 | 0.843 | 106.9 | 0.813 |
|  |  |  |  |  |  | 277.6 | 0.991 | 88.6 | 0.733 |
|  |  |  |  |  |  |  |  | 70.4 | 0.740 |
|  |  |  |  |  |  |  |  | 52.1 | 0.873 |
|  |  |  |  |  |  |  |  | 33.8 | 0.944 |
|  |  |  |  |  |  |  |  | 15.5 | 0.643 |
|  |  |  |  |  |  |  |  | 357.3 | 0.250 |
|  |  |  |  |  |  |  |  | 339.0 | 0.178 |
|  |  |  |  |  |  |  |  | 320.7 | 0.375 |
|  |  |  |  |  |  |  |  | 302.4 | 0.726 |
|  |  |  |  |  |  |  |  | 284.2 | 0.929 |

Time Domain Alternative Method:

Time domain techniques can also be used to validate the spatial correlation. The spatial correlation validation measurement setup is illustrated in Figure C.3.4-3. In this case a Signal generator transmits a CW signal through the MIMO test system. The signal is received by a test antenna within the test area. Finally, the signal is collected by a signal analyzer and the measured signal is stored for postprocessing.

Figure C.3.4-3: Configuration for spatial correlation validation based on time domain techniques

For each spatial point, the channel emulator should issue a trigger signal each time fading is started. For each point collect a time domain trace with the signal analyzer, when done, stop fading. Data recording is synchronized with the channel emulator trigger.

Follow the same procedure to postprocess the data and calcalate the spatial correlation by setting m to 1. The settings for the Signal Generator and Signal Analyzer are in Table C.3.4-6 and C.3.4-7 respectively.

Table C.3.4-6: Signal Generator Settings

| Item | Unit | Value |
| --- | --- | --- |
| Center frequency | MHz | Downlink center frequency in Table C.3.1-1 |
| Output power | dBm | Function of the CE. Sufficiently above Noise Floor |

Table C.3.4-7: Signal Analyzer Settings

| Item | Unit | Value |
| --- | --- | --- |
| Center frequency | MHz | Downlink center frequency in Table C.3.1-1 |
| Sampling | Hz | At least 15 times bigger than the max Doppler spread (fd=v/λ) |
| Observation time | s | At least 16s. Channel Model length should be the same or greater than the observation time. |

C.3.5	Cross-polarization

This measurement checks how well the measured vertically or horizontally polarized power levels follow expected values. The test setup for cross-polarization is the same as PDP validation in Figure C.3.2-1.

Method of measurement: 		Step the emulation and store traces from VNA.

VNA settings:

Table C.3.5-1: VNA settings for cross-polarization

| Item | Unit | Value |
| --- | --- | --- |
| Center frequency | MHz | Downlink Center Frequency in Table C.3.1-1 |
| Span | MHz | 40 |
| Number of traces |  | 1000 |
| Number of points |  | 802 |
| Averaging |  | 1 |

Channel model specification:

Table C.3.5-2: Channel model specification for cross-polarization.

| Item | Unit | Value |
| --- | --- | --- |
| Center frequency | MHz | Downlink center frequency in Table C.3.1-1 |
| Distance between traces in channel model | wavelength (Note) | > 2 |
| Channel model |  | As specified in Annex C.1 |
| Mobile speed | km/h | 30 |
| NOTE: Time [s] = distance [] / MS speed [/s] MS speed [/s] = MS speed [m /s] / Speed of light [m/s] * Center frequency [Hz] |  |  |

Measurement Procedure:

1.	Play or step through the channel model listed in Annex C.1.

2.	Measure the absolute power received at the center of the test zone, averaged over a statistically significant number of fades.

a.	Use a vertically polarized sleeve dipole to measure the V component.

b.	Use a horizontally polarized (vertically oriented) magnetic loop dipole, or a horizontally polarized sleeve dipole measured in four orthogonal horizontal positions and summed to measure the H component.

3.	Calculate the V/H ratio.

4.	Compare it with the theory value.

C.3.6	Power validation

This measurement checks the total power in the center of the test zone. The power validation is measured with a spectrum analyzer as shown in Figure C.3.6-1.

Figure C.3.6-1: Setup for power validation measurements

Spectrum analyzer settings:

Table C.3.6-1: Spectrum analyzer settings for power validation measurements

| Item | Unit | Value |
| --- | --- | --- |
| Center frequency | MHz | Downlink center frequency in Table C.3.1-1 |
| Integrated Channel Span | Hz | 20MHz |
| RBW | Hz | 30 kHz |
| VBW | Hz | ≥10MHz |
| Number of points |  | ≥400 |
| Averaging |  | ≥100 |
| Detector |  | RMS |

Measurement Procedure:

1. Place a vertical reference dipole in the center of the test zone connected to a spectrum analyzer (or power meter) via a cable.

2. Record the cable and reference dipole gains.

3. Load the target channel model into the channel emulator.

4. Start the NR FR1 signaling in the base station emulator with the required parameter identical to the measurements conditions.

5. Average the power received by the spectrum analyzer for a sufficient amount of time to account for the fading channel – one full channel simulation might be unnecessary.

6. Repeat steps 1 to 4 with a magnetic loop for the horizontal polarization, or a horizontally polarized sleeve dipole measured in four orthogonal horizontal positions and summed to measure the H component.

7. Calculate the total power received at the test area as the sum of the power in the two polarizations.

C.4	Validation Pass/fail limit

Annex D (normative):
<FR2 Channel models and Validation procedure >

< Editor’s note: Detailed structure of the subclause is TBD.

Include FR2 channel model tables after down-selection for requirement, FR2 channel model validation procedure, and validation pass/fail requirement.>

Annex E (normative):
<gNB configurations>

< Editor’s note: Detailed structure of the subclause is TBD.

Final RMC after down-selection for requirement, both FR1 and FR2. >

E.1	FR1 gNB configurations

The gNodeB emulator parameters shall be set according to Table E.1-1 for FR1 common parameters, Table E.1-2 for FR1 FDD 2x2 test parameters, Table E.1-3 for FR1 TDD 2x2 test parameters, Table E.1-4 for FR1 FDD 4x4 test parameters, and Table E.1-5 for FR1 TDD 4x4 test parameters.

Table E.1-1: FR1 Common test parameters

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  | Transmission scheme 1 |  |
| Carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 2) |  | RBs | 0 |  |
|  | Subcarrier spacing |  | kHz | 15 or 30 |  |
| DL BWP configuration #1 | Cyclic prefix |  |  | Normal |  |
|  | RB offset |  | RBs | 0 |  |
|  | Number of contiguous PRB |  | PRBs | Maximum transmission bandwidth configuration as specified in clause 5.3.2 of TS 38.101-1 for tested channel bandwidth and subcarrier spacing |  |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |  |
|  | SSB position in burst |  |  | First SSB in Slot #0 |  |
|  | SSB periodicity |  | ms | 20 |  |
|  | First DMRS position for Type A PDSCH mapping |  |  | 2 |  |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | Each slot |  |
|  | Symbols with PDCCH |  | Symbols | 0, 1 |  |
|  | Number of PRBs in CORESET |  |  | Table 5.2-2 of TS 38.101-4 for tested channel bandwidth and subcarrier spacing |  |
|  | Number of PDCCH candidates and aggregation levels |  |  | 1/AL8 |  |
|  | CCE-to-REG mapping type |  |  | Non-interleaved |  |
|  | DCI format |  |  | 1_1 |  |
|  | TCI state |  |  | TCI state #1 |  |
| Cross carrier scheduling |  |  |  | Not configured |  |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resource 1,2,3,4 |  |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |  |
|  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |  |
|  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |  |
|  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2,3,4 |  |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |  |
|  | CSI-RS offset |  | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |  |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |  |
|  | QCL info |  |  | TCI state #0 |  |
| NZP CSI-RS for CSI acquisition | First subcarrier index in the PRB used for CSI-RS |  |  | k0 = 0 |  |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 12 |  |
|  | Number of CSI-RS ports (X) |  |  | Same as number of transmit antenna |  |
|  | CDM Type |  |  | ‘FD-CDM2’ |  |
|  | Density (ρ) |  |  | 1 |  |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 30 kHz SCS: 40 |  |
|  | CSI-RS offset |  | Slots | 0 |  |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |  |
|  | QCL info |  |  | TCI state #1 |  |
| ZP CSI-RS for CSI acquisition | First subcarrier index in the PRB used for CSI-RS |  |  | k0 = 4 |  |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 12 |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |
|  | CDM Type |  |  | ‘FD-CDM2’ |  |
|  | Density (ρ) |  |  | 1 |  |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 30 kHz SCS: 40 |  |
|  | CSI-RS offset |  | Slots | 0 |  |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |  |
| PDSCH DMRS configuration | Antenna ports indexes |  |  | {1000, 1001} for Rank 2 tests {1000-1003} for Rank 4 tests |  |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  | 1 for Rank 2 tests 2 for Rank 4 tests |  |
| TCI state #0 | Type 1 QCL information | SSB index |  | SSB #0 |  |
|  |  | QCL Type |  | Type C |  |
|  | Type 2 QCL information | SSB index |  | N/A |  |
|  |  | QCL Type |  | N/A |  |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from ‘CSI-RS for tracking’ configuration |  |
|  |  | QCL Type |  | Type A |  |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |  |
|  |  | QCL Type |  | N/A |  |
| PT-RS configuration |  |  |  | PT-RS is not configured |  |
| Maximum number of code block groups for ACK/NACK feedback |  |  |  | 1 |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |
| HARQ ACK/NACK bundling |  |  |  | Multiplexed |  |
| Redundancy version coding sequence |  |  |  | N.A |  |
| Precoding configuration |  |  |  | SP Type I, Random per slot with PRB bundling granularity |  |
| Symbols for all unused REs |  |  |  | OCNG Annex A.5 of TS 38.101-4 |  |
| Minimum Number of Slots per Stream |  |  |  | 20000 for 15kHz SCS 40000 for 30kHz SCS |
| Note 1: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission. Note 2: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 for tested channel bandwidth and subcarrier spacing. |  |  |  |  |  |

Table E.1-2: Test parameters for FR1 FDD 2x2

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Reference channel |  |  | R.PDSCH.1-3.1 FDD (Note 1) |
| Bandwidth |  | MHz | 10 |
| SCS |  | kHz | 15 |
| Modulation DL |  |  | 64QAM |
| Modulation UL |  |  | QPSK |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 20 |
|  | CSI-RS offset | Slots | Table 8.2-1. |
| Number of HARQ Processes |  |  | 1 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |
| Note 1: “R.PDSCH.1-3.1 FDD” is defined in Table A.3.2.1.1-3 of TS 38.101-4 |  |  |  |

Table E.1-3: Test parameters for FR1 TDD 2x2

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Reference channel |  |  | R.PDSCH.2-3.1 TDD (Note 1) |
| Bandwidth |  | MHz | 40 |
| SCS |  | kHz | 30 |
| Modulation DL |  |  | 64QAM |
| Modulation UL |  |  | QPSK |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS |  | Table 8.2-1. |
|  | CSI-RS periodicity | Slots | 40 |
|  | CSI-RS offset | Slots | Table 8.2-1. |
| Number of HARQ Processes |  |  | 1 |
| TDD UL-DL pattern |  |  | FR1.30-1 (Note 2) |
| Note 1: “R.PDSCH.2-3.1 TDD” is defined in Table A.3.2.2.2-3 of TS 38.101-4 Note 2: “FR1.30-1” is defined in Annex A.1.2 of TS 38.101-4 |  |  |  |

Table E.1-4: Test parameters for FR1 FDD 4x4

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Reference channel |  |  | R.PDSCH.1-2.4 FDD (Note 1) |
| Bandwidth |  | MHz | 10 |
| SCS |  | kHz | 15 |
| Modulation DL |  |  | 16QAM |
| Modulation UL |  |  | QPSK |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 20 |
|  | CSI-RS offset | Slots | Table 8.2-1. |
| Number of HARQ Processes |  |  | 1 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |
| Note 1: “R.PDSCH.1-2.4 FDD” is defined in Table A.3.2.1.1-2 of TS 38.101-4 |  |  |  |

Table E.1-5: Test parameters for FR1 TDD 4x4

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Reference channel |  |  | R.PDSCH.2-2.4 TDD (Note 1) |
| Bandwidth |  | MHz | 40 |
| SCS |  | kHz | 30 |
| Modulation DL |  |  | 16QAM |
| Modulation UL |  |  | QPSK |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS |  | Table 8.2-1. |
|  | CSI-RS periodicity | Slots | 40. |
|  | CSI-RS offset | Slots | Table 8.2-1. |
| Number of HARQ Processes |  |  | 1 |
| TDD UL-DL pattern |  |  | FR1.30-1 (Note 2) |
| Note 1: “R.PDSCH.2-2.4 TDD” is defined in Table A.3.2.2.2-2 of TS 38.101-4 Note 2: “FR1.30-1” is defined in Annex A.1.2 of TS 38.101-4 |  |  |  |

E.2	FR2 gNB configurations

Annex F (normative):
<Environmental requirements>

< Editor’s note: Detailed structure of the subclause is TBD.

Refer to TR38.827 with normal condition. both FR1 and FR2 >

Annex G (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-08 | RAN4#96-e | R4-2012709 |  |  |  | Initial Skeleton | 0.0.1 |
| 2020-11 | RAN4#97-e | R4-2016216 |  |  |  | R4-2016218, TP to TS 38.151 v0.0.1 on general part R4-2016222, TP to TS 38.151 v0.0.1 on FR1 test system for requirements R4-2017584, TP to TS 38.151 v0.0.1 on FR1 Channel model and RMC | 0.1.0 |