3GPP TS 38.323 V1.0.1 (2017-10)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

NR;

Packet Data Convergence Protocol (PDCP) specification

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword, …]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	7

4	General	7

4.1	Introduction	7

4.2	Architecture	7

4.2.1	PDCP structure	7

4.2.2	PDCP entities	8

4.3	Services	9

4.3.1	Services provided to upper layers	9

4.3.2	Services expected from lower layers	9

4.4	Functions	10

5	Procedures	10

5.1	PDCP entity handling	10

5.1.1	PDCP entity establishment	10

5.1.2	PDCP entity re-establishment	10

5.1.3	PDCP entity release	11

5.2	Data transfer	11

5.2.1	Transmit operation	11

5.2.2	Receive operation	12

5.3	SDU discard	12

5.4	Status reporting	12

5.4.1	Transmit operation	12

5.4.2	Receive operation	13

5.5	Data recovery	13

5.6	Data volume calculation	13

5.7	Header compression and decompression	13

5.7.1	Supported header compression protocols and profiles	13

5.7.2	Configuration of header compression	14

5.7.3	Protocol parameters	14

5.7.4	Header compression	14

5.7.5	Header decompression	15

5.7.6	PDCP Control PDU for interspersed ROHC feedback	15

5.7.6.1	Transmit Operation	15

5.7.6.2	Receive Operation	15

5.8	Ciphering and deciphering	15

5.9	Integrity protection and verification	15

5.10	Handling of unknown, unforeseen, and erroneous protocol data	16

6	Protocol data units, formats, and parameters	16

6.1	Protocol data units	16

6.1.1	Data PDU	16

6.1.2	Control PDU	16

6.2	Formats	16

6.2.1	General	16

6.2.2	Data PDU	17

6.2.2.1	Data PDU for SRBs	17

6.2.2.2	Data PDU for DRBs with 12 bits PDCP SN	17

6.2.2.3	Data PDU for DRBs with 18 bits PDCP SN	17

6.2.3	Control PDU	18

6.2.3.1	Control PDU for PDCP status report	18

6.2.3.2	Control PDU for interspersed ROHC feedback	18

6.3	Parameters	19

6.3.1	General	19

6.3.2	PDCP SN	19

6.3.3	Data	19

6.3.4	MAC-I	19

6.3.5	COUNT	19

6.3.6	R	19

6.3.7	D/C	20

6.3.8	PDU type	20

6.3.9	FMC	20

6.3.10	Bitmap	20

6.3.11	Interspersed ROHC feedback	20

7	State variables, constants, and timers	20

7.1	State variables	20

7.2	Constants	21

7.3	Timers	22

Annex A (informative): Change history	22

Annex B (informative; to be removed later):  Agreements related to PDCP specification	22

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

1	Scope

The present document provides the description of the Packet Data Convergence Protocol (PDCP).

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.300: "NG Radio Access Network; Overall description".

[3]	3GPP TS 38.331: "NR Radio Resource Control (RRC); Protocol Specification".

[4]	3GPP TS 38.321: "NR Medium Access Control (MAC) protocol specification".

[5]	3GPP TS 38.322: "NR Radio Link Control (RLC) protocol specification".

[6]	3GPP TS 33.501: "Security Architecture and Procedures for 5G System ".

[7]	IETF RFC 5795: "The RObust Header Compression (ROHC) Framework".

[8]	IETF RFC 3095: "RObust Header Compression (ROHC): Framework and four profiles: RTP, UDP, ESP and uncompressed".

[9]	IETF RFC 4815: "RObust Header Compression (ROHC): Corrections and Clarifications to RFC 3095".

[10]	IETF RFC 6846: "RObust Header Compression (ROHC): A Profile for TCP/IP (ROHC-TCP)".

[11]	IETF RFC 5225: "RObust Header Compression (ROHC) Version 2: Profiles for RTP, UDP, IP, ESP and UDP Lite”.

3	Definitions and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in [1].

AM DRB: a data radio bearer which utilizes RLC AM.

Non-split bearer: a bearer whose radio protocols are located in either the MgNB or the SgNB to use MgNB or SgNB resource, respectively.

PDCP data volume: the amount of data available for transmission in a PDCP entity.

Split bearer: in dual connectivity, a bearer whose radio protocols are located in both the MgNB and the SgNB to use both MgNB and SgNB resources.

UM DRB: a data radio bearer which utilizes RLC UM.

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in [1].

AM	Acknowledged Mode

ARP	Address Resolution Protocol

CID	Context Identifier

DRB	Data Radio Bearer carrying user plane data

gNB	NR Node B

HFN	Hyper Frame Number

IETF	Internet Engineering Task Force

IP	Internet Protocol

L2	Layer 2 (data link layer)

L3	Layer 3 (network layer)

MAC	Medium Access Control

MAC-I	Message Authentication Code for Integrity

MCG	Master Cell Group

NG-RAN	NR Radio Access Network

PDCP	Packet Data Convergence Protocol

PDU	Protocol Data Unit

RB	Radio Bearer

RFC	Request For Comments

RLC	Radio Link Control

ROHC	RObust Header Compression

RRC	Radio Resource Control

RTP	Real Time Protocol

SAP	Service Access Point

SCG	Secondary Cell Group

SDU	Service Data Unit

SN	Sequence Number

SRB	Signalling Radio Bearer carrying control plane data

TCP	Transmission Control Protocol

UDP	User Datagram Protocol

UE	User Equipment

UM	Unacknowledged Mode

X-MAC	Computed MAC-I

4	General

4.1	Introduction

The present document describes the functionality of the PDCP.

4.2	Architecture

4.2.1	PDCP structure

Figure 4.2.1.1 represents one possible structure for the PDCP sublayer; it should not restrict implementation. The figure is based on the radio interface protocol architecture defined in [2].

Figure 4.2.1-1: PDCP layer, structure view

The PDCP sublayer is configured by upper layers [3]. The PDCP sublayer is used for RBs mapped on DCCH and DTCH type of logical channels. The PDCP sublayer is not used for any other type of logical channels.

Each RB (except for SRB0) is associated with one PDCP entity. Each PDCP entity is associated with one, two, or four RLC entities depending on the RB characteristic (e.g uni-directional/bi-directional or split/non-split) or RLC mode. For non-split bearers, each PDCP entity is associated with one UM RLC entity, two UM RLC entities (one for each direction), or one AM RLC entity. For split bearers, each PDCP entity is associated with two UM RLC entities (for same direction), four UM RLC entities (two for each direction), or two AM RLC entities (for same direction).

4.2.2	PDCP entities

The PDCP entities are located in the PDCP sublayer. Several PDCP entities may be defined for a UE. Each PDCP entity is carrying the data of one radio bearer.

A PDCP entity is associated either to the control plane or the user plane depending on which radio bearer it is carrying data for.

Figure 4.2.2.1 represents the functional view of the PDCP entity for the PDCP sublayer; it should not restrict implementation. The figure is based on the radio interface protocol architecture defined in [2].

For split bearers, routing or duplication is performed in the transmitting PDCP entity.

Figure 4.2.2-1: PDCP layer, functional view

4.3	Services

4.3.1	Services provided to upper layers

The PDCP layer provides its services to the RRC or SDAP layers. The following services are provided by PDCP to upper layers:

-	transfer of user plane data;

-	transfer of control plane data;

-	header compression;

-	ciphering;

-	integrity protection.

The maximum supported size of a PDCP SDU is 9000 bytes. The maximum supported size of a PDCP Control PDU is 9000 bytes.

4.3.2	Services expected from lower layers

A PDCP entity expects the following services from lower layers per RLC entity (for a detailed description see [5]):

-	acknowledged data transfer service, including indication of successful delivery of PDCP PDUs;

-	unacknowledged data transfer service.

4.4	Functions

The PDCP layer supports the following functions:

-	transfer of data (user plane or control plane);

-	maintenance of PDCP SNs;

-	header compression and decompression using the ROHC protocol;

-	ciphering and deciphering;

-	integrity protection and integrity verification;

-	timer based SDU discard;

-	for split bearers, routing or duplication;

-	reordering and in-order delivery;

-	duplicate discarding;

Editor’s Note: Duplication function is not complete and is targeted for completion in June 2018.

5	Procedures

5.1	PDCP entity handling

5.1.1	PDCP entity establishment

When upper layers request a PDCP entity establishment for a radio bearer, the UE shall:

-	establish a PDCP entity for the radio bearer;

-	set the state variables of the PDCP entity to initial values;

-	follow the procedures in subclause 5.2.

5.1.2	PDCP entity re-establishment

When upper layers request a PDCP entity re-establishment, the UE shall additionally perform once the procedures described in this section. After performing the procedures in this section, the UE shall follow the procedures in subclause 5.2.

When upper layers request a PDCP entity re-establishment, the transmitting PDCP entity shall:

-	for UM DRBs and AM DRBs, reset the header compression protocol for uplink and start with an IR state in U-mode [8] [9] if drb-ContinueROHC is not configured [3];

-	for UM DRBs and SRBs, set TX_NEXT to the initial value;

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs;

-	apply the ciphering algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	apply the integrity protection algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	for UM DRBs, for each PDCP SDU already associated with a PDCP SN but for which a corresponding PDU has not previously been submitted to lower layers:

-	consider the PDCP SDUs as received from upper layer;

-	perform transmission of the PDCP SDUs in ascending order of the COUNT value associated to the PDCP SDU prior to the PDCP re-establishment without restarting the discardTimer.

-	for AM DRBs, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, perform retransmission or transmission of all the PDCP SDUs already associated with PDCP SNs in ascending order of the COUNT values associated to the PDCP SDU prior to the PDCP entity re-establishment as specified below:

-	perform header compression of the PDCP SDU as specified in the subclause 5.7.4;

-	perform integrity protection and ciphering of the PDCP SDU using the COUNT value associated with this PDCP SDU as specified in the subclause 5.9 and 5.8;

-	submit the resulting PDCP Data PDU to lower layer.

When upper layers request a PDCP entity re-establishment, the receiving PDCP entity shall:

-	process the PDCP Data PDUs that are received from lower layers due to the re-establishment of the lower layers, as specified in the subclause 5.2.2.1;

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs

-	for UM DRBs, if t-Reordering is running:

- 	stop and reset t-Reordering;

-	deliver all stored PDCP SDUs to the upper layers in ascending order of associated COUNT values after performing header decompression;

-    for AM DRBs, perform header decompression for all stored PDCP SDUs if drb-ContinueROHC is not configured [3];

-	for UM DRBs and AM DRBs, reset the header compression protocol for downlink and start with NC state in U-mode [8] [9] if drb-ContinueROHC is not configured [3];

-	for UM DRBs and SRBs, set RX_NEXT and RX_DELIV to the initial value;

-	apply the ciphering algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	apply the integrity protection algorithm and key provided by upper layers during the PDCP entity re-establishment procedure.

5.1.3	PDCP entity release

When upper layers request a PDCP entity release for a radio bearer, the UE shall:

-	for UM DRBs and AM DRBs, deliver the PDCP SDUs stored in the receiving PDCP entity to upper layers in ascending order of associated COUNT values after performing header decompression, if not decompressed before;

-	release the PDCP entity for the radio bearer.

5.2	Data transfer

5.2.1	Transmit operation

At reception of a PDCP SDU from upper layers, the transmitting PDCP entity shall:

-	start the discardTimer associated with this PDCP SDU (if configured);

For a PDCP SDU received from upper layers, the transmitting PDCP entity shall:

-	associate the COUNT value corresponding to TX_NEXT to this PDCP SDU;

NOTE:	Associating more than half of the PDCP SN space of contiguous PDCP SDUs with PDCP SNs, when e.g., the PDCP SDUs are discarded or transmitted without acknowledgement, may cause HFN desynchronization problem. How to prevent HFN desynchronization problem is left up to UE implementation.

-	perform header compression of the PDCP SDU as specified in the subclause 5.7.4;

-	perform integrity protection, and ciphering using the TX_NEXT as specified in the subclause 5.9 and 5.8, respectively;

-	set the PDCP SN of the PDCP Data PDU to TX_NEXT modulo 2[pdcp-SN-Size];

-	increment TX_NEXT by one;

-	submit the resulting PDCP Data PDU to lower layer as specified below.

When submitting a PDCP Data PDU to lower layer, the transmitting PDCP entity shall:

-	if the transmitting PDCP entity is associated with one RLC entity:

-	submit the PDCP Data PDU to the associated RLC entity;

-	else, if the transmitting PDCP entity is associated with two RLC entities:

-	if pdcpDuplication is configured and activated:

-	duplicate the PDCP Data PDU and submit the PDCP Data PDU to both associated RLC entities;

-	else, if pdcpDuplication is configured but not activated:

-	submit the PDCP Data PDU to the configured RLC entity;

-	else:

-	if the total amount of PDCP data volume and RLC data volume pending for initial transmission in the two associated RLC entities is less than ul-DataSplitThreshold:

-	submit the PDCP Data PDU to the configured RLC entity;

-	else:

-	submit the PDCP Data PDU to one of the associated RLC entity.

Editor’s Note: The exact data submission procedure needs further discussion. It is FFS when the PDCP entity submits the PDCP Data PDU to lower layer, FFS how may PDCP PDUs the PDCP entity can submit to lower layer, and FFS what is compared with threshold.

5.2.2	Receive operation

In this section, following definitions are used:

-	HFN(State Variable): the HFN part (i.e. the number of most significant bits equal to HFN length) of the State Variable.

-	SN(State Variable): the SN part (i.e. the number of least significant bits equal to PDCP SN length) of the State Variable.

-	RCVD_SN: the PDCP SN of the received PDCP Data PDU, included in the PDU header.

-	RCVD_HFN: the HFN of the received PDCP Data PDU, calculated by the receiving PDCP entity.

-	RCVD_COUNT: the COUNT of the received PDCP Data PDU = [RCVD_HFN, RCVD_SN].

5.2.2.1	Actions when a PDCP Data PDU is received from lower layers

At reception of a PDCP Data PDU from lower layers, the receiving PDCP entity shall determine the COUNT value of the received PDCP Data PDU, i.e. RCVD_COUNT, as follows:

-	if RCVD_SN < SN(RX_DELIV) – Window_Size:

-	RCVD_HFN = HFN(RX_DELIV) + 1;

-	else if RCVD_SN >= SN(RX_DELIV) + Window_Size:

-	RCVD_HFN = HFN(RX_DELIV) – 1;

-	else:

-	RCVD_HFN = HFN(RX_DELIV);

-	RCVD_COUNT = [RCVD_HFN, RCVD_SN].

After determining the COUNT value of the received PDCP Data PDU = RCVD_COUNT, the receiving PDCP entity shall:

-	if RCVD_COUNT < RX_DELIV; or

-	if the PDCP Data PDU with COUNT = RCVD_COUNT has been received before:

-	perform deciphering and integrity verification of the PDCP Data PDU using COUNT = RCVD_COUNT;

-	if integrity verification fails:

-	indicate the integrity verification failure to upper layer;

-	discard the PDCP Data PDU;

-	else:

-	perform deciphering and integrity verification of the PDCP Data PDU using COUNT = RCVD_COUNT;

-	if integrity verification fails:

-	indicate the integrity verification failure to upper layer;

-	discard the PDCP Data PDU;

If the received PDCP Data PDU with COUNT value = RCVD_COUNT is not discarded above, the receiving PDCP entity shall:

-	store the resulting PDCP SDU in the reception buffer;

-	if RCVD_COUNT >= RX_NEXT:

-	update RX_NEXT to RCVD_COUNT + 1;

-	if outOfOrderDelivery is configured:

-	deliver the resulting PDCP SDU to upper layers;

-	if RCVD_COUNT = RX_DELIV:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from COUNT = RX_DELIV;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers, with COUNT value > RX_DELIV;

-	if t-Reordering is running, and if RX_DELIV >= RX_REORD:

-	stop and reset t-Reordering;

-	if t-Reordering is not running (includes the case when t-Reordering is stopped due to actions above), and RX_DELIV < RX_NEXT:

-	update RX_REORD to RX_NEXT;

-	start t-Reordering.

5.2.2.2	Actions when a t-Reordering expires

When t-Reordering expires, the receiving PDCP entity shall:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before:

-	all stored PDCP SDU(s) with associated COUNT value(s) < RX_REORD;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from RX_REORD;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers, with COUNT value >= RX_REORD;

-	if RX_DELIV < RX_NEXT:

-	update RX_REORD to RX_NEXT;

-	start t-Reordering.

5.2.2.3	Actions when the value of t-Reordering is reconfigured

When the value of the t-Reordering is reconfigured by upper layers while the t-Reordering is running, the UE shall:

-	update RX_REORD to RX_NEXT;

-	stop and restart t-Reordering.

5.3	SDU discard

When the discardTimer expires for a PDCP SDU, or the successful delivery of a PDCP SDU is confirmed by PDCP status report, the transmitting PDCP entity shall discard the PDCP SDU along with the corresponding PDCP Data PDU. If the corresponding PDCP Data PDU has already been submitted to lower layers, the discard is indicated to lower layers.

NOTE:	Discarding a PDCP SDU already associated with a PDCP SN causes a SN gap in the transmitted PDCP Data PDUs, which increases PDCP reordering delay in the receiving PDCP entity. It is up to UE implementation how to minimize SN gap after SDU discard.

5.4	Status reporting

5.4.1	Transmit operation

For AM DRBs configured by upper layers to send a PDCP status report in the uplink (statusReportRequired [3]), the receiving PDCP entity shall trigger a PDCP status report when:

-	upper layer requests a PDCP entity re-establishment;

-	upper layer requests a PDCP data recovery;

If a PDCP status report is triggered, the receiving PDCP entity shall:

-	compile a PDCP status report as indicated below by:

-	setting the FMC field to RX_DELIV;

-	if RX_DELIV < RX_NEXT:

-	allocating a Bitmap field of length in bits equal to the number of COUNTs from and not including the first missing PDCP SDU up to and including the last out-of-sequence PDCP SDUs, rounded up to the next multiple of 8, or up to and including a PDCP SDU for which the resulting PDCP Control PDU size is equal to 9000 bytes, whichever comes first;

-	setting in the bitmap field as ‘0’ for all PDCP SDUs that have not been received, and optionally PDCP SDUs for which decompression have failed;

-	setting in the bitmap field as ‘1’ for all PDCP SDUs that have been received;

-	submit the PDCP status report to lower layers as the first PDCP PDU for transmission.

5.4.2	Receive operation

For AM DRBs, when a PDCP status report is received in the downlink, the transmitting PDCP entity shall:

-	consider for each PDCP SDU, if any, with the bit in the bitmap set to '1', or with the associated COUNT value less than the value of FMC field as successfully delivered, and discard the PDCP SDU as specified in the subclause 5.3.

5.5	Data recovery

For AM DRBs, when upper layers request a PDCP data recovery for a radio bearer, the transmitting PDCP entity shall:

-	if the radio bearer is configured by upper layers to send a PDCP status report in the uplink (statusReportRequired [3]), transmit a PDCP status report as described in subclause 5.4.1;

-	perform retransmission of all the PDCP Data PDUs previously submitted to re-established AM RLC entity in ascending order of the associated COUNT values from the first PDCP Data PDU for which the successful delivery has not been confirmed by lower layers.

After performing the above procedures, the transmitting PDCP entity shall follow the procedures in subclause 5.2.1.

5.6	Data volume calculation

For the purpose of MAC buffer status reporting, the transmitting PDCP entity shall consider the following as PDCP data volume:

-	the PDCP SDUs for which no PDCP Data PDUs have been constructed;

-	the PDCP Data PDUs that have not been submitted to lower layers;

-	the PDCP Control PDUs;

-	for AM DRBs, the PDCP SDUs to be retransmitted according to subclause 5.1.2;

-	for AM DRBs, the PDCP Data PDUs to be retransmitted according to subclause 5.5.

5.7	Header compression and decompression

5.7.1	Supported header compression protocols and profiles

The header compression protocol is based on the Robust Header Compression (ROHC) framework [7]. There are multiple header compression algorithms, called profiles, defined for the ROHC framework. Each profile is specific to the particular network layer, transport layer or upper layer protocol combination e.g. TCP/IP and RTP/UDP/IP.

The detailed definition of the ROHC channel is specified as part of the ROHC framework [7]. This includes how to multiplex different flows (header compressed or not) over the ROHC channel, as well as how to associate a specific IP flow with a specific context state during initialization of the compression algorithm for that flow.

The implementation of the functionality of the ROHC framework and of the functionality of the supported header compression profiles is not covered in this specification.

In this version of the specification the support of the following profiles is described:

Table 5.7.1-1: Supported header compression protocols and profiles

| Profile Identifier | Usage | Reference |
| --- | --- | --- |
| 0x0000 | No compression | RFC 5795 |
| 0x0001 | RTP/UDP/IP | RFC 3095, RFC 4815 |
| 0x0002 | UDP/IP | RFC 3095, RFC 4815 |
| 0x0003 | ESP/IP | RFC 3095, RFC 4815 |
| 0x0004 | IP | RFC 3843, RFC 4815 |
| 0x0006 | TCP/IP | RFC 6846 |
| 0x0101 | RTP/UDP/IP | RFC 5225 |
| 0x0102 | UDP/IP | RFC 5225 |
| 0x0103 | ESP/IP | RFC 5225 |
| 0x0104 | IP | RFC 5225 |

5.7.2	Configuration of header compression

PDCP entities associated with DRBs can be configured by upper layers [3] to use header compression. Each PDCP entity carrying user plane data may be configured to use header compression. In this version of the specification, only the robust header compression protocol (ROHC) is supported. Every PDCP entity uses at most one ROHC compressor instance and at most one ROHC decompressor instance.

5.7.3	Protocol parameters

RFC 5795 has configuration parameters that are mandatory and that must be configured by upper layers between compressor and decompressor peers [7]; these parameters define the ROHC channel. The ROHC channel is a unidirectional channel, i.e. if headerCompression is configured there is one channel for the downlink and one for the uplink, and if uplinkOnlyHeaderCompression is configured there is only one channel for the uplink. There is thus one set of parameters for each channel, and if headerCompression is configured the same values shall be used for both channels belonging to the same PDCP entity.

These parameters are categorized in two different groups, as defined below:

-	M:	Mandatory and configured by upper layers.

-	N/A: Not used in this specification.

The usage and definition of the parameters shall be as specified below.

-	MAX_CID (M): This is the maximum CID value that can be used. One CID value shall always be reserved for uncompressed flows. The parameter MAX_CID is configured by upper layers (maxCID [3]).

-	LARGE_CIDS: This value is not configured by upper layers, but rather it is inferred from the configured value of MAX_CID according to the following rule:

-	If MAX_CID > 15 then LARGE_CIDS = TRUE else LARGE_CIDS = FALSE.

-	PROFILES (M): Profiles are used to define which profiles are allowed to be used by the UE. The list of supported profiles is described in section 5.7.1. The parameter PROFILES is configured by upper layers (profiles for uplink and downlink [3]).

-	FEEDBACK_FOR (N/A): This is a reference to the channel in the opposite direction between two compression endpoints and indicates to what channel any feedback sent refers to. Feedback received on one ROHC channel for this PDCP entity shall always refer to the ROHC channel in the opposite direction for this same PDCP entity.

-	MRRU (N/A): ROHC segmentation is not used.

5.7.4	Header compression

If header compression is configured, the header compression protocol generates two types of output packets:

-	compressed packets, each associated with one PDCP SDU

-	standalone packets not associated with a PDCP SDU, i.e. interspersed ROHC feedback

A compressed packet is associated with the same PDCP SN and COUNT value as the related PDCP SDU.

Interspersed ROHC feedback are not associated with a PDCP SDU. They are not associated with a PDCP SN and are not ciphered.

NOTE:	If the MAX_CID number of ROHC contexts are already established for the compressed flows and a new IP flow does not match any established ROHC context, the compressor should associate the new IP flow with one of the ROHC CIDs allocated for the existing compressed flows or send PDCP SDUs belonging to the IP flow as uncompressed packet.

5.7.5	Header decompression

If header compression is configured by upper layers for PDCP entities associated with user plane data, the PDCP Data PDUs are decompressed by the header compression protocol after performing deciphering as explained in the subclause 5.8.

5.7.6	PDCP Control PDU for interspersed ROHC feedback

5.7.6.1	Transmit Operation

When an interspersed ROHC feedback is generated by the header compression protocol, the transmitting PDCP entity shall:

-	submit to lower layers the corresponding PDCP Control PDU as specified in subclause 6.2.3.2 i.e. without associating a PDCP SN, nor performing ciphering.

5.7.6.2	Receive Operation

At reception of a PDCP Control PDU for interspersed ROHC feedback from lower layers, the receiving PDCP entity shall:

-	deliver the corresponding interspersed ROHC feedback to the header compression protocol without performing deciphering.

5.8	Ciphering and deciphering

The ciphering function includes both ciphering and deciphering and is performed in PDCP, if configured. The data unit that is ciphered is the data part of the PDCP Data PDU (see subclause 6.3.3) and the MAC-I (see subclause 6.3.4). The ciphering is not applicable to PDCP Control PDUs.

The ciphering algorithm and key to be used by the PDCP entity are configured by upper layers [3] and the ciphering method shall be applied as specified in [6].

The ciphering function is activated by upper layers [3]. When security is activated, the ciphering function shall be applied to all PDCP Data PDUs indicated by upper layers [3] for the downlink and the uplink, respectively.

For downlink and uplink ciphering and deciphering, the parameters that are required by PDCP for ciphering are defined in [6] and are input to the ciphering algorithm. The required inputs to the ciphering function include the COUNT value, and DIRECTION (direction of the transmission: set as specified in [6]).The parameters required by PDCP which are provided by upper layers [3] are listed below:

-	BEARER (defined as the radio bearer identifier in [6]. It will use the value RB identity –1 as in [3]);

-	KEY (the ciphering keys for the control plane and for the user plane are [KRRCenc] and [KUPenc], respectively).

Editor’s Note: Exact name of ciphering keys are FFS.

5.9	Integrity protection and verification

The integrity protection function includes both integrity protection and integrity verification and is performed in PDCP, if configured. The data unit that is integrity protected is the PDU header and the data part of the PDU before ciphering. The integrity protection is always applied to PDCP Data PDUs of SRBs. The integrity protection is applied to PDCP Data PDUs of DRBs for which integrity protection is configured. The integrity protection is not applicable to PDCP Control PDUs.

The integrity protection algorithm and key to be used by the PDCP entity are configured by upper layers [3] and the integrity protection method shall be applied as specified in [6].

The integrity protection function is activated by upper layers [3]. When security is activated, the integrity protection function shall be applied to all PDUs including and subsequent to the PDU indicated by upper layers [3] for the downlink and the uplink, respectively.

NOTE:	As the RRC message which activates the integrity protection function is itself integrity protected with the configuration included in this RRC message, this message needs first be decoded by RRC before the integrity protection verification could be performed for the PDU in which the message was received.

For downlink and uplink integrity protection and verification, the parameters that are required by PDCP for integrity protection are defined in [6] and are input to the integrity protection algorithm. The required inputs to the integrity protection function include the COUNT value, and DIRECTION (direction of the transmission: set as specified in [6]). The parameters required by PDCP which are provided by upper layers [3] are listed below:

-	BEARER (defined as the radio bearer identifier in [6]. It will use the value RB identity –1 as in [3]);

-	KEY ([KRRCint]).

Editor’s Note: Exact name of integrity key is FFS.

At transmission, the UE computes the value of the MAC-I field and at reception it verifies the integrity of the PDCP Data PDU by calculating the X-MAC based on the input parameters as specified above. If the calculated X-MAC corresponds to the received MAC-I, integrity protection is verified successfully.

5.10	Handling of unknown, unforeseen, and erroneous protocol data

When a PDCP PDU that contains reserved or invalid values is received, the receiving PDCP entity shall:

-	discard the received PDU.

6	Protocol data units, formats, and parameters

6.1	Protocol data units

6.1.1	Data PDU

The PDCP Data PDU is used to convey one or more of followings:

-	a PDCP SN;

-	user plane data;

-	control plane data;

-	a MAC-I.

6.1.2	Control PDU

The PDCP Control PDU is used to convey one of followings:

-	a PDCP status report;

-	an interspersed ROHC feedback.

6.2	Formats

6.2.1	General

A PDCP PDU is a bit string that is byte aligned (i.e. multiple of 8 bits) in length. In the figures in subclause 6.2, bit strings are represented by tables in which the most significant bit is the leftmost bit of the first line of the table, the least significant bit is the rightmost bit on the last line of the table, and more generally the bit string is to be read from left to right and then in the reading order of the lines. The bit order of each parameter field within a PDCP PDU is represented with the first and most significant bit in the leftmost bit and the last and least significant bit in the rightmost bit.

PDCP SDUs are bit strings that are byte aligned (i.e. multiple of 8 bits) in length. A compressed or uncompressed SDU is included into a PDCP Data PDU from the first bit onward.

6.2.2	Data PDU

6.2.2.1	Data PDU for SRBs

Figure 6.2.2.1-1 shows the format of the PDCP Data PDU with 12 bits PDCP SN. This format is applicable for SRBs.

Figure 6.2.2.1-1: PDCP Data PDU format for SRBs

6.2.2.2	Data PDU for DRBs with 12 bits PDCP SN

Figure 6.2.2.2-1 shows the format of the PDCP Data PDU with 12 bits PDCP SN. This format is applicable for UM DRBs and AM DRBs.

Figure 6.2.2.2-1: PDCP Data PDU format with 12 bits PDCP SN

6.2.2.3	Data PDU for DRBs with 18 bits PDCP SN

Figure 6.2.2.3-1 shows the format of the PDCP Data PDU with 18 bits PDCP SN. This format is applicable for UM DRBs and AM DRBs.

Figure 6.2.2.3-1: PDCP Data PDU format for DRBs with 18 bits PDCP SN

6.2.3	Control PDU

6.2.3.1	Control PDU for PDCP status report

Figure 6.2.3.1-1 shows the format of the PDCP Control PDU carrying one PDCP status report. This format is applicable for AM DRBs.

Figure 6.2.3.1-1: PDCP Control PDU format for PDCP status report

6.2.3.2	Control PDU for interspersed ROHC feedback

Figure 6.2.3.2-1 shows the format of the PDCP Control PDU carrying one interspersed ROHC feedback. This format is applicable for UM DRBs and AM DRBs.

Figure 6.2.3.2-1: PDCP Control PDU format for interspersed ROHC feedback

6.3	Parameters

6.3.1	General

If not otherwise mentioned in the definition of each field then the bits in the parameters shall be interpreted as follows: the left most bit string is the first and most significant and the right most bit is the last and least significant bit.

Unless otherwise mentioned, integers are encoded in standard binary encoding for unsigned integers. In all cases the bits appear ordered from MSB to LSB when read in the PDU.

6.3.2	PDCP SN

Length: 12, or 18 bits as indicated in table 6.3.2.1. The length of the PDCP SN is configured by upper layers (pdcp-SN-Size [3]).

Table 6.3.2-1: PDCP SN length

| Length | Description |
| --- | --- |
| 12 | UM DRBs, AM DRBs, and SRBs |
| 18 | UM DRBs, and AM DRBs |

6.3.3	Data

Length: Variable

This field includes one of the followings:

-	Uncompressed PDCP SDU (user plane data, or control plane data);

-	Compressed PDCP SDU (user plane data only).

6.3.4	MAC-I

Length: 32 bits

This field carries a message authentication code calculated as specified in subclause 5.9.

For SRBs, the MAC-I field is always present. If integrity protection is not configured, the MAC-I field is still present but should be padded with padding bits set to 0.

For DRBs, the MAC-I field is present only when the DRB is configured with integrity protection.

6.3.5	COUNT

Length: 32 bits

The COUNT value is composed of a HFN and the PDCP SN. The size of the HFN part in bits is equal to 32 minus the length of the PDCP SN.

Figure 6.3.5-1: Format of COUNT

NOTE:	COUNT does not wrap around.

6.3.6	R

Length: 1 bit

Reserved. In this version of the specification reserved bits shall be set to 0. Reserved bits shall be ignored by the receiver.

6.3.7	D/C

Length: 1 bit

This field indicates whether the corresponding PDCP PDU is a PDCP Data PDU or a PDCP Control PDU.

Table 6.3.7-1: D/C field

| Bit | Description |
| --- | --- |
| 0 | Control PDU |
| 1 | Data PDU |

6.3.8	PDU type

Length: 3 bits

This field indicates the type of control information included in the corresponding PDCP Control PDU.

Table 6.3.8-1: PDU type

| Bit | Description |
| --- | --- |
| 000 | PDCP status report |
| 001 | Interspersed ROHC feedback |
| 010-111 | Reserved |

6.3.9	FMC

Length: 32 bits

First Missing COUNT. This field indicates the COUNT value of the first missing PDCP SDU within the reordering window, i.e. RX_DELIV.

6.3.10	Bitmap

Length: Variable. The length of the bitmap field can be 0.

This field indicates which SDUs are missing and which SDUs are correctly received in the receiving PDCP entity. The bit position of Nth bit in the Bitmap is N, i.e., the bit position of the first bit in the Bitmap is 1.

Table 6.3.10-1 Bitmap

| Bit | Description |
| --- | --- |
| 0 | PDCP SDU with COUNT = (FMC + bit position) modulo 232 is missing. |
| 1 | PDCP SDU with COUNT = (FMC + bit position) modulo 232 is correctly received. |

6.3.11	Interspersed ROHC feedback

Length: Variable

This field contains one ROHC packet with only feedback, i.e. a ROHC packet that is not associated with a PDCP SDU as defined in subclause 5.7.4.

7	State variables, constants, and timers

7.1	State variables

This sub clause describes the state variables used in PDCP entities in order to specify the PDCP protocol. The state variables defined in this subclause are normative.

All state variables are non-negative integers, and take values from 0 to [232 – 1].

PDCP Data PDUs are numbered integer sequence numbers (SN) cycling through the field: 0 to [2[pdcp-SN-Size] – 1].

The transmitting PDCP entity shall maintain the following state variables:

a)	TX_NEXT

This state variable indicates the COUNT value of the next PDCP SDU to be transmitted. The initial value is 0.

The receiving PDCP entity shall maintain the following state variables:

a)	RX_NEXT

This state variable indicates the COUNT value of the next PDCP SDU expected to be received. The initial value is 0.

b)	RX_DELIV

This state variable indicates the COUNT value of the first PDCP SDU not delivered to the upper layers, but still waited for. The initial value is 0.

c)	RX_REORD

This state variable indicates the COUNT value following the COUNT value associated with the PDCP Data PDU which triggered t-Reordering.

7.2	Constants

a) Window_Size

This constant indicates the size of the reordering window. The value equals to 2[pdcp-SN-Size] – 1.

7.3	Timers

The transmitting PDCP entity shall maintain the following timers:

a) discardTimer

This timer is configured only for DRBs. The duration of the timer is configured by upper layers [3]. In the transmitter, a new timer is started upon reception of an SDU from upper layer.

The receiving PDCP entity shall maintain the following timers:

b) t-Reordering

The duration of the timer is configured by upper layers [3]. This timer is used to detect loss of PDCP Data PDUs as specified in the subclause 5.2.2. If t-Reordering is running, t-Reordering shall not be started additionally, i.e. only one t-Reordering per receiving PDCP entity is running at a given time.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | Version |
| 2017.03 | RAN2#97bis | R2-1703512 | - | - | - | First version. | x.y.z |
| 2017.04 | RAN2#97bis | R2-1703916 | - | - | - | Change section name “Retransmission” to “Data recovery” | 0.0.1 |
| 2017.05 | RAN2#98 | R2-1704076 | - | - | - | Initial draft TS capturing outcome of e-mail discussion [97bis#24] | 0.0.5 |
| 2017.06 | RAN2 NR AH | R2-1706868 | - | - | - | Capture agreements made in RAN2#98 | 0.1.0 |
| 2017.08 | RAN2 NR AH | R2-1707507 | - | - | - | Capture agreements made in RAN2 NR AH#2 | 0.2.0 |
| 2017.08 | RAN2#99 | R2-1709097 | - | - | - | Adding integrity protection in section 5.1.2 | 0.2.1 |
| 2017.08 | RAN2#99 | R2-1709753 | - | - | - | Capture agreements made in RAN2#99 | 0.3.0 |
| 2017.09 | RANP#77 | RP-171993 | - | - | - | Provided for information to RAN | 1.0.0 |
| 2017.10 | RAN2#99bis | R2-17xxxxx | - | - | - | Capture agreements made in RAN2#99bis | 1.0.1 |

Annex B (informative; to be removed later): 
Agreements related to PDCP specification

Editor’s Note: This section collects the agreements that may have impact on PDCP specification. This section will be removed when the PDCP specification is officially released.

NR AH #1 (Jan. 2017)

-	PDCP and RLC layers have separate PDCP SN and RLC SN.

-	Packet duplication is supported for user plane and control plane in NR-PDCP (This agreement does not preclude discussion of other mechanisms to improve mobility robustness)

-	The PDCP function in the transmitter supports packet duplication and the PDCP function in the receiver supports duplicate packet removal.

RAN2#97 (Athens, Feb. 2017)

-	Split bearers support RLC UM mode besides RLC AM mode.

-	For DL and UL, duplication solution for CA case uses PDCP duplication to more than 1 logical channel so that the duplicated PDCP PDUs are sent over different carriers.

RAN2#97bis (Spokane, Mar. 2017)

-	Duplicate detection and discard functionalities for SRBs should be introduced in LTE PDCP to support duplication via split SRB in LTE-NR tight interworking scenarios where LTE is the MN.

-	RRC configures PDCP for duplication and the radio protocols of the UE with separate RLC entities and logical channels to handle duplicates (referred to as “legs”)

-	only one additional leg is configured for PDCP duplicates.

-	In NR, SRBs (except SRB0) should be always mapped on RLC AM.

-	In NR, DRB can be mapped on either RLC AM or RLC UM.

-	Confirmed that RoHC will remain in PDCP.

-	We will not consider concatenation in PDCP or SDAP further in Rel-15.

-	We will not look at TCP optimisations further in Rel-15.

2-	  handling of handover for AM mode:

-	1: LTE- like handover

-	2: No Key change, Data Recovery, RLC re-establishment

-	2 handling of handover for UM mode:

-	1: LTE- like handover

-	2: No Key change, RLC re-established

-	2 handling of handover for SRB:

-	1: LTE- like handover

-	2: No Key change, RLC re-established

-	NR PDCP aims at defining a single procedure for reception for a AM and UM, as well as DRB and SRB (will be concluded in stage 3 discussions)

-	NR PDCP procedures are based on COUNT instead of SN, which is determined in the beginning of the procedure. All NR PDCP state variables are based on COUNT.

-	Adopt modulo operation for arithmetic operation.

-	Some PDCP recovery mechanism based of PDCP status report is supported at least for the handover case and DC.  FFS if there are other cases in which this may be performed.

-	PDCP SN length should be up to at least up 18 bits (for data bearer). FFS if there are use cases in which larger value is needed and which value for AM/UM.

-	PDCP SN length 12 bits is supported for RLC UM and AM and for both DRB(s) and SRB(s)

-	LTE PDCP PDU format for DRBs will be the reused

-	NR should support jumbo frame (9KB)

-	FFS NR UE can support super jumbo frame (65KB) and is optional.

-	PDCP reordering should always enabled for SRBs

-	PDCP duplicate discarding should be done at least when duplication is enabled for SRBs/DRBs(s).

-	A unified re-ordering schemes is used for DRB(s)/SRB(s) and UM and AM, with LTE as baseline.

-	It is desirable to disable PDCP reordering.  FFS how to signal it

-	Use First Missing COUNT (FMC) instead of FMS in the PDCP Status Report.

-	PDCP SDU is discarded upon the expiry of PDCP discard timer.

-	PDCP SDU is discarded when successful delivery is confirmed by PDCP status report.

RAN2#98 (Hangzhou, May. 2017)

-	UL PDCP duplication is configurable per DRB and, for NR-NR DC case, per SRB.

-	FFS whether the initial state of the UL PDCP duplication (duplication active or not active and if not active which leg is used) is a default or whether the initial state can be signalled by RRC

-	RAN2 will attempt to define at least one mechanism to start/stop PDCP duplication more quickly and with less signalling overhead compared to RRC reconfiguration.

-	MAC CE approach will be used for control of UL duplication. Optimisations to reliability of the MAC CE will not be introduced for this mechanism. No optimisations or additional interactions between network nodes are introduced for this mechanism.

-	Duplicated PDCP PDUs are submitted to two different RLC entities

-	Lets try to use short and more descriptive names

-	Duplicate detection in RLC UM is not necessary

-	Option 2 will be implemented (for COUNT determination)

-	Agree to TP additions (ROHC profiles for PDCP)

-	the D/C field is not present in the PDCP data PDU format for SRB.

-	the D/C field is present in the PDCP data PDU format for DRB.

-	the P field is not present in the PDCP data PDU format for SRB and as a baseline for DRB as well.

-	the D/C field is present in all PDCP control PDUs.

-	the PDU type field is present in all PDCP control PDUs.

-	the PDU type field is 3 bits.

-	RAN2 assumes that out-of-order/duplicated reception of ROHC feedback is not an issue to be resolved to RAN2.

-	Out-of-order/duplicated reception of PDCP status report is not an issue to be resolved in RAN2.

-	As a baseline, PDCP SN number will not be added to the PDCP Control PDUs

-	12 and 18 bit PDCP SN is used for NR

-	A configurable threshold approach is used to determine if the UE should transmit on one more than one link.   As a baseline, the buffer status is used as a threshold.  FFS if other thresholds like data rates or delay can be considered.

-	If below a threshold the UE transmits on one link.  When above the threshold enhancements can be considered to allow for pre-processing and link performance.

-	As a baseline no specification impacts are needed to handle ROHC operation.

-	The need to disable reordering and how is postponed until receive window operation is completed

NR AH #2 (Jun. 2017)

-	The UE and gNB shall support integrity protection of user data. (SA3 agreement)

-	Super jumbo frames are not supported.

-	PUSH window with text in R2-1706869 is supported and endorsed.

-	For outdated and duplicated case the UE does not decipher or decompress.

-	Out-of-order delivery is configurable by RRC.

-	Out of order delivery does not affect window status, i.e. RX_DELIV.

-	Integrity verification, if applicable, is applied at reception of a PDU. Following LTE, it is applied also for PDUs received out of the window and/or duplicates (that are already stored).

-	The LTE threshold based mechanism is used for UL bearer split.

-	Pre-processing is allowed in the split bearer case, similar to single carrier case.  How much pre-processing is done is left to UE implementation.

-	PDCP should ensure that not more than half PDCP SN space is allocated.

-	As a baseline, lossless SN re-configuration is not supported for RLC AM.

RAN2#99 (Berlin, Aug. 2017)

-	EN-DC capable UE without EN-DC operation configured can be configured with NR PDCP version for SRBs and DRBs.

-	NR-PDCP is used for ‘MCG split or duplicate SRB’.

-	Non split SRB1 and 2 can be transmitted over either LTE-PDCP or NR-PDCP, which is configurable by network (at least after initial connection establishment).

-	PDCP/RLC is re-established if security key is changed for the bearer. (Maybe revisited is a solution for avoiding MAC reset is selected and the solution is suitable for avoiding PDCP/RLC reset)

-	FFS: Whether PDCP is re-established for the bearer type change between MCG bearer and SCG bearer if NR PDCP is used for MCG bearer.  CA duplication is supported for all non-split UM DRBs if the bearer uses NR-PDCP, for all architecture options (apart from cases excluded by 1 and 2)

-	FFS: for AM DRBs and SRBs

-	DC duplication is supported for all split DRB and SRBs if the bearer uses NR-PDCP, for all architecture options

-	We will not introduce new bearer type changes into RRC, but user plane session can discuss and decide terminology for DC duplication, CA duplication, split bearer operation, etc (Some clarification is needed for how to handle CA duplication within the current bearer type change discussion)

-	Priority in user plane session for addressing the stage 3 details:

-	1: UM for DRBs with CA and DC duplication; SRBs (AM) with DC duplication;

-	2: SRBs (AM) with CA duplication

-	3: AM for DRBs with DC duplication

-	4: AM for DRBs with CA duplication

-	UP integrity protection can be configured on a per radio bearer (i.e. per DRB) basis.

-	For DC, when DRB duplication is deactivated via MAC CE, the UE falls back to the split bearer operation.  Once de-activated we rely on split bearer operation and configuration.

-	MAC-I field is not present for a DRB for which integrity protection is not configured.

-	Unless SA3 raises some concerns, the maximum supported size of a PDCP SDU is 9000 bytes.

-	In NR, PDCP status report length should be limited, same as maximum length of PDCP SDU length.

-	Upon PDCP re-establishment, for UM DRBs the receiving PDCP entity shall deliver to upper layers any stored SDUs previously received (like also done at PDCP-entity release).

-	ROHC context continue is applied for AM DRB

-	The header compression should not be configured when out-of-order delivery is allowed for PDCP SDU.

-	Out-of-order delivery is configured only when the radio bearer is established (added to the field description of RRC).

-	RX_DELIV definition should be updated such as “new RX_DELIV = old RX_DELIV+1”.

-	A UE with split bearer can be configured to transmit on a single path via RRC signalling.

-	The UE is allowed to pre-process data for split bearer before a request from lower layers is received and is allowed to submit to lower layers before a request is received.  A restriction on bad UE behaviour or a requirement on proper behaviour will be added.  FFS how to capture it (e.g.  capture how avoid bad UE behaviours related to which PDCP SN are sent to the RLC and not transmitted at the end and whether and how to capture a pre-processing limit)

RAN2#99bis (Prague, Oct. 2017)

Agreements from UP session

-	PDCP SN length can be separately configured for UL and DL

-	R2-1711575 PDCP specification updates: Agree to add PDCP Data Volume

-	R2-1711575 PDCP specification updates: Keep the duplication and put a editors note that duplication is to be completed.  No further optimization to the duplication.

-	R2-1710903 NR PDCP COUNT length: In NR, the length of PDCP COUNT is 32-bits.

-	R2-1711577 Support for out-of-order delivery in PDCP: the TP is endorsed

-	R2-1711673 Handling of COUNT wrap around:	We will update the note “when performing comparison of values related to COUNT, the UE takes into account that COUNT is a 32-bit value, which may wrap around (e.g., COUNT value of 232 - 1 is less than COUNT value of 0).” So that it doesn’t state the Count may wrap around.

-	A note to provide guidance to the UE will be added (e.g. the UE should minimize transmission gap among the UL split bearer)

-	When comparing with the PDCP split threshold the UE should take into account the PDCP data volume and RLC pre-processed data (e.g. pending data for transmission).  This is will be added in normative text.

-	FFS if there is any issue on BSR reporting on the secondary leg.

-	R2-1711545 Supporting UL single path transmission in PDCP: The rapporteur will capture the changes once the RRC signalling has been completed.

-	For AM DRBs, upon PDCP re-establishment, if drb-ContinueROHC is not configured, the receiving PDCP entity performs header decompression for stored PDCP PDUs before header decompression reset

-	For EN-DC, for RLC UM PDCP entity processes PDCP Data PDUs that are received from lower layers due to the re-establishment of the lower layers, at PDCP re-establishment.

-	UL only ROHC for TCP/IP profile is supported in NR as in LTE

-	R2-1711576 TP on PDCP data volume calculation: Update the first bullet to “for which no PDCP Data PDU has been constructed”

-	R2-1711576 TP on PDCP data volume calculation: The first part of the TP is agreeable and will be added to the rapporteur running TS.

-	For each DRB, the enabling of integrity protection should be configured by RRC signaling semi-statically.  The enabling of integrity protection for one DRB can only be changed using a HO procedure.

-	For the data plane PDCP PDU, the presence of MAC-I field can be derived based on the RRC configuration, thus no MAC-I presence indicator is needed.  The current PDCP spec already implements this.

-	R2-1710781 Solutions for SN gap issue due to PDCP discard: we will leave it to UE implemtation

-	PDCP data recovery procedure is not defined for UM DRBs.

-	R2-1711653 PDCP retransmissions upon UL path change & re-establishment: FFS on UE behaviour upon UL path switch (e.g. retransmissions and data recovery)

-	MAC-CE based leg switching is not supported for split bearer in Rel-15

Agreements from main session

-	Trigger conditions for PDCP recovery will be captured in RRC spec.

-	For DRBs, change from LTE PDCP to NR PDCP is done (via handover) using a release and add of the DRB (in a single message) or full configuration.

-	No need to specify behaviour for PDCP count wrap around in NR (network expected to take action before this happens)

-	PDCP recovery does not apply to SRBs