3GPP TR 38.716-01-01 V0.8.0 (2019-11)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Networks;

NR intra band Carrier Aggregation (CA) Rel-16 for xCC Down Link (DL) / yCC Up Link (UL) including contiguous and non-contiguous spectrum (x >= y)

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2016, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	6

2	References	14

3	Definitions, symbols and abbreviations	14

3.1	Definitions	14

3.2	Symbols	14

3.3	Abbreviations	15

4	Background	15

4.1	TR Maintenance	15

5	Intra-Band Contiguous Carrier Aggregation FR1: Specific Band Combination Part	16

5.1	CA_2DL_n66B_1UL_n66A	16

5.1.1	Channel bandwidths per operating band for CA	16

5.1.2	UE co-existence studies	17

5.2	CA_2DL_n71B	17

5.2.1	Channel bandwidths per operating band for CA	17

5.2.2	UE co-existence studies	17

5.2.3	REFSENS	17

5.3	CA_2DL_n41C_1UL_n41A	18

5.3.1	Channel bandwidths per operating band for CA	18

5.3.2	Co-existence studies	19

5.4	CA_2DL_n48B, CA_2DL_n48C	20

5.4.1	Channel bandwidths per operating band for CA	20

5.4.2	Co-existence studies	21

5.4.3	REFSENS	21

5.5	CA_2DL_n1B_ 1UL_n1A	22

5.5.1	Operating band for CA	22

5.5.2	Channel bandwidths per operating band for CA	22

5.5.3	Co-existence studies	22

5.6	CA_2DL_n7B_ 1UL_n7A	23

5.6.1	Operating band for CA	23

5.6.2	Channel bandwidths per operating band for CA	23

5.6.3	Co-existence studies	23

6	Intra-Band Non-Contiguous Carrier Aggregation FR1: Specific Band Combination Part	24

6.1	CA_2DL_n66(2A)_1UL_n66A	24

6.1.1	Channel bandwidths per operating band for CA	24

6.1.2	UE co-existence studies	25

6.1.3	REFSENS	25

6.2	CA_2DL_n41(2A)_1UL_n41A	25

6.2.2	Channel bandwidths per operating band for CA	26

6.2.3	Co-existence studies	27

6.2.4	REFSENS	27

6.3	CA_2DL_n25(2A)_1UL_n25A	28

6.3.1	Channel bandwidths per operating band for CA	28

6.3.2	Co-existence studies	28

6.3.3	REFSENS	28

6.4	CA_2DL_n48(2A)_1UL_n48A	30

6.4.1	Channel bandwidths per operating band for CA	30

6.4.2	Co-existence studies	30

6.4.3	REFSENS	30

6.5	CA_2DL_n3(2A)_ 1UL_n3A	31

6.5.1	Operating band for CA	31

6.5.2	Channel bandwidths per operating band for CA	31

6.5.3	Co-existence studies	31

6.5.4	REFSENS	31

6.6	CA_2DL_n7(2A)_ 1UL_n7A	32

6.6.1	Operating band for CA	32

6.6.2	Channel bandwidths per operating band for CA	32

6.6.3	Co-existence studies	33

6.6.4	REFSENS	33

7.1	CA_xDL_n257a_xUL_n257a (x=2, 3, 4, 5, 6, 7, 8, a=G, H, I, J, K, L, M)	34

7.1.1	Channel bandwidths per operating band for CA	34

7.1.2	UE co-existence studies	35

7.2	CA_n258	36

7.2.1	Operating bands for CA	36

7.2.2	Channel bandwidths per operating band for CA	37

7.2.3	Co-existence studies	39

8	Intra-Band Non-Contiguous Carrier Aggregation FR2: Specific Band Combination Part	39

8.1	Intra band non-contiguous CA configurations n260	39

8.2	Intra band non-contiguous CA fallback groups n260	43

8.3	Intra band non-contiguous CA configurations n261	67

8.4	Intra band non-contiguous CA fallback groups n261	70

Annex A: Change history	79

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is a technical report for NR Intra-band Carrier Aggregation Rel-16 for xDL/yUL including contiguous and non-contiguous spectrum under Rel-16 time frame. The purpose is to gather the relevant background information and studies in order to address NR Intra-band Carrier Aggregation requirements for the Rel-16 band combinations in Table 1-1, Table 1-2, Table 1-3 and Table 1-4.

Table 1-1: Release 16 NR Intra-band carrier contiguous aggregation combinations FR1

CA combination

DL_n78C_UL_n78C

DL_n79C_UL_n79C

DL_n66B_UL_n66A

DL_n41C_UL_n41A

DL_n41C_UL_n41C

DL_n71B

DL_n77C_UL_n77C

2CC_DL_n5B_BCS0

2CC_DL_n5B_1CC_UL_n5A_BCS0

2CC_DL_n5B_2CC _UL_n5B_BCS0

2CC_DL_n48B_BCS0

2CC_DL_n48B_1CC_UL_n48A_BCS0

2CC_DL_n48B_2CC_UL_n48B_BCS0

2CC_DL_n48C_BCS0

2CC_DL_n48C_1CC _UL_n48A_BCS0

2CC_DL_n48C_2CC _UL_n48C_BCS0

2CC_DL_n66B_BCS0

2CC_DL_n66B_1CC_UL_n66A_BCS0

2CC_DL_n66B_2CC_UL_n66B_BCS0

DL_n3B_UL_n3B_BCS0

CA_n1B_UL_n1B_BCS0

CA_n41C_UL_n41A_BCS1

CA_n7B_UL_n7A_BCS0

CA_n7B_UL_n7B_BCS0

Table 1-2: Release 16 NR Intra-band carrier non-contiguous aggregation combinations FR1

CA combination

DL_n66(2A)_UL_66A

DL_n41(2A)_UL_n41A

DL_n41(2A)_UL_n41(2A)

DC_n25(2A)_UL_n25A

2CC_DL_n2(2A)_BCS0

2CC_DL_n2(2A)_1CC_UL_n2A_BCS0

2CC_DL_n5(2A)_BCS0

2CC_DL_n5(2A)_1CC_UL_n5A_BCS0

2CC_DL_n48(2A)_BCS0

2CC_DL_n48(2A)_1CC _n48A_BCS0

2CC_DL_n66(2A)_1CC_UL_n66A_BCS0

3CC_DL_ n66(A-B)_BCS0

3CC_DL_ n66(A-B)_1CC _UL_n66A_BCS0

3CC_DL_ n66(A-B)_2CC _UL_n66B_BCS0

CA_n41(2A)

DL_n77(2A)_UL_n77A

DL_n78(2A)_UL_n78A

CA_n25(2A) UL_n25A_BCS0

CA_n7(2A)_UL_n7A_BCS0

CA_n78(2A)_UL_n78A_BCS0

CA_n3(2A)_UL_n3A_BCS0

CA_n41(2A)_UL_n41A_BCS1

CA_n78(2A)_UL_n78(2A)_BCS0

Table 1-3: Release 16 NR Intra-band carrier contiguous aggregation combinations FR2

| CA combination | REL-indep. from |
| --- | --- |
| CA_n258B | Rel-15 |
| CA_n258C | Rel-15 |
| CA_n258D | Rel-15 |
| CA_n258E | Rel-15 |
| CA_n258F | Rel-15 |
| CA_n258G | Rel-15 |
| CA_n258H | Rel-15 |
| CA_n258I | Rel-15 |
| CA_n258J | Rel-15 |
| CA_n258K | Rel-15 |
| CA_n258L | Rel-15 |
| CA_n258M | Rel-15 |
| CA_n257G_UL_n257G | Rel-15 |
| CA_n257H_UL_n257G | Rel-15 |
| CA_n257H_UL_n257H | Rel-15 |
| CA_n257I_UL_n257G | Rel-15 |
| CA_n257I_UL_n257H | Rel-15 |
| CA_n257I_UL_n257I | Rel-15 |
| CA_n257J_UL_n257G | Rel-15 |
| CA_n257J_UL_n257H | Rel-15 |
| CA_n257J_UL_n257I | Rel-15 |
| CA_n257J_UL_n257J | Rel-15 |
| CA_n257K_UL_n257G | Rel-15 |
| CA_n257K_UL_n257H | Rel-15 |
| CA_n257K_UL_n257I | Rel-15 |
| CA_n257K_UL_n257J | Rel-15 |
| CA_n257K_UL_n257K | Rel-15 |
| CA_n257L_UL_n257G | Rel-15 |
| CA_n257L_UL_n257H | Rel-15 |
| CA_n257L_UL_n257I | Rel-15 |
| CA_n257L_UL_n257J | Rel-15 |
| CA_n257L_UL_n257K | Rel-15 |
| CA_n257L_UL_n257L | Rel-15 |
| CA_n257M_UL_n257G | Rel-15 |
| CA_n257M_UL_n257H | Rel-15 |
| CA_n257M_UL_n257I | Rel-15 |
| CA_n257M_UL_n257J | Rel-15 |
| CA_n257M_UL_n257K | Rel-15 |
| CA_n257M_UL_n257L | Rel-15 |
| CA_n257M_UL_n257M | Rel-15 |
| CA_n261G_UL_n261G | Rel-15 |
| CA_n261H_UL_n261H | Rel-15 |
| CA_n261I_UL_n261H | Rel-15 |
| CA_n261J_UL_n261H | Rel-15 |
| CA_n261K_UL_n261H | Rel-15 |
| CA_n261L_UL_n261H | Rel-15 |
| CA_n261M_UL_n261H | Rel-15 |
| CA_n258D_UL_n258D | Rel-15 |
| CA_n257C | Rel-15 |

Table 1-4: Release 16 NR Intra-band carrier non-contiguous aggregation combinations FR2

| CA combination | REL-indep. from |
| --- | --- |
| CA_n260(5A) | Rel-15 |
| CA_n260(6A) | Rel-15 |
| CA_n260(7A) | Rel-15 |
| CA_n260(8A) | Rel-15 |
| CA_n260(9A) | Rel-15 |
| CA_n260(10A) | Rel-15 |
| CA_n260(2G) | Rel-15 |
| CA_n260(4G) | Rel-15 |
| CA_n260(2H) | Rel-15 |
| CA_n260(2O) | Rel-15 |
| CA_n260(3O) | Rel-15 |
| CA_n260(4O) | Rel-15 |
| CA_n260(2P) | Rel-15 |
| CA_n260(4P) | Rel-15 |
| CA_n261(2D) | Rel-15 |
| CA_n261(2G) | Rel-15 |
| CA_n261(3G) | Rel-15 |
| CA_n261(4G) | Rel-15 |
| CA_n261(2H) | Rel-15 |
| CA_n261(2I) | Rel-15 |
| CA_n261(2O) | Rel-15 |
| CA_n261(4O) | Rel-15 |
| CA_n261(7O) | Rel-15 |
| CA_n261(2P) | Rel-15 |
| CA_n261(4Q) | Rel-15 |
| CA_n261(7O) | Rel-15 |
| CA_n261(2P) | Rel-15 |
| CA_n261(4Q) | Rel-15 |
| CA_n260(2P) | Rel-15 |
| CA_n260(3G) | Rel-15 |
| CA_n260(4G) | Rel-15 |
| CA_n260(A-G-O) | Rel-15 |
| CA_n260(2A-G-O) | Rel-15 |
| CA_n260(2A-2G-O) | Rel-15 |
| CA_n260(A-G) | Rel-15 |
| CA_n260(G-O) | Rel-15 |
| CA_n260(G-O) | Rel-15 |
| CA_n260(A-D) | Rel-15 |
| CA_n260(2A-D) _UL_n260(2A) | Rel-15 |
| CA_n260(A-D-O) | Rel-15 |
| CA_n260(2A-D-O) | Rel-15 |
| CA_n260(D-2O) | Rel-15 |
| CA_n260(A-D-2O) | Rel-15 |
| CA_n260(2A-D-2O) | Rel-15 |
| CA_n260(A-2D) | Rel-15 |
| CA_n260(2A-2D) | Rel-15 |
| DC_n260(A-P) | Rel-15 |
| DC_n260(2A-P)_UL_n260(2A) | Rel-15 |
| DC_n260(2A-O)_UL_n260(2A) | Rel-15 |
| DC_n260(2A-G)_UL_n260(2A) | Rel-15 |
| DC_n260(2A-H)_UL_n260(2A) | Rel-15 |
| DC_n260(A-2P) | Rel-15 |
| DC_n260(2A-2P) | Rel-15 |
| CA_n260(3A-3O) | Rel-15 |
| CA_n260(D-2G) | Rel-15 |
| CA_n260(2D-O) | Rel-15 |
| CA_n260(G-2O) | Rel-15 |
| CA_n260(2G-2O) | Rel-15 |
| CA_n260(G-3O) | Rel-15 |
| CA_n260(2G-3O) | Rel-15 |
| CA_n260(G-4O) | Rel-15 |
| CA_n260(2G-4O) | Rel-15 |
| CA_n260(3G-O) | Rel-15 |
| CA_n260(4G-O) | Rel-15 |
| CA_n260(H-O) | Rel-15 |
| CA_n260(2H-O) | Rel-15 |
| CA_n261(A-D) | Rel-15 |
| CA_n261(A-D-H) | Rel-15 |
| CA_n261(A-G) | Rel-15 |
| CA_n261(A-G-H) | Rel-15 |
| CA_n261(G-I) | Rel-15 |
| CA_n261(A-G-I) | Rel-15 |
| CA_n261(A-H-I) | Rel-15 |
| CA_n261(G-H) | Rel-15 |
| CA_n261(H-I) | Rel-15 |
| CA_n260(2A-G-2O) | Rel-15 |
| CA_n260(A-2G-2O) | Rel-15 |
| CA_n260(2A-2O-P) | Rel-15 |
| CA_n260(2A-O-2P) | Rel-15 |
| CA_n260(A-2O-2P) | Rel-15 |
| CA_n260(2A-2O-Q) | Rel-15 |
| CA_n260(2A-O-2Q) | Rel-15 |
| CA_n260(A-2O-2Q) | Rel-15 |
| CA_n260(4A-3O) | Rel-15 |
| CA_n260(3A-4O) | Rel-15 |
| CA_n260(4A-Q) | Rel-15 |
| CA_n260(3A-2Q) | Rel-15 |
| CA_n260(3A-P) | Rel-15 |
| CA_n260(A-O-P) | Rel-15 |
| CA_n260(A-Q) | Rel-15 |
| CA_n260(P-Q) | Rel-15 |
| CA_n260(2A-3P) | Rel-15 |
| CA_n260(A-4P) | Rel-15 |
| CA_n260(6A-O) | Rel-15 |
| CA_n260(5A-2O) | Rel-15 |
| CA_n260(5A-3O) | Rel-15 |
| CA_n260(6A-P) | Rel-15 |
| CA_n260(5A-2P) | Rel-15 |
| CA_n260(8A-O) | Rel-15 |
| CA_n260(7A-2O) | Rel-15 |
| CA_n260(2O-P) | Rel-15 |
| CA_n260(O-2P) | Rel-15 |
| CA_n261(A-D-O) | Rel-15 |
| CA_n261(A-2O) | Rel-15 |
| CA_n261(D-2O) | Rel-15 |
| CA_n261(A-2G-O) | Rel-15 |
| CA_n261(A-G-2O) | Rel-15 |
| CA_n261(2G-2O) | Rel-15 |
| CA_n261(A-3G) | Rel-15 |
| CA_n261(A-2G-O) | Rel-15 |
| CA_n261(3G-O) | Rel-15 |
| CA_n261(A-3G) | Rel-15 |
| CA_n261(A-3O) | Rel-15 |
| CA_n261(A-6O) | Rel-15 |
| CA_n261(A-P) | Rel-15 |
| CA_n261(A-Q) | Rel-15 |
| CA_n260(A-G-2O) | Rel-15 |
| CA_n260(2A-O-P) | Rel-15 |
| CA_n260(A-2O-P) | Rel-15 |
| CA_n260(A-O-2P) | Rel-15 |
| CA_n260(A-2O-P) | Rel-15 |
| CA_n260(2A-O-Q) | Rel-15 |
| CA_n260(A-2O-Q) | Rel-15 |
| CA_n260(2A-2Q) | Rel-15 |
| CA_n260(A-O-2Q) | Rel-15 |
| CA_n260(2O-2Q) | Rel-15 |
| CA_n260(3A-Q) | Rel-15 |
| CA_n260(O-P) | Rel-15 |
| CA_n260(A-3P) | Rel-15 |
| CA_n260(5A-O) | Rel-15 |
| CA_n260(5A-P) | Rel-15 |
| CA_n260(4A-2P) | Rel-15 |
| CA_n260(7A-O) | Rel-15 |
| CA_n261(A-O) | Rel-15 |
| CA_n261(A-2G) | Rel-15 |
| CA_n261(A-G-O) | Rel-15 |
| CA_n261(2G-O) | Rel-15 |
| CA_n261(G-2O) | Rel-15 |
| CA_n261(3O) | Rel-15 |
| CA_n261(A-5O) | Rel-15 |
| CA_n261(6O) | Rel-15 |
| CA_n260(2A-Q) | Rel-15 |
| CA_n260(A-O-Q) | Rel-15 |
| CA_n260(2O-Q) | Rel-15 |
| CA_n260(A-2Q) | Rel-15 |
| CA_n260(O-2Q) | Rel-15 |
| CA_n260(4A-P) | Rel-15 |
| CA_n260(3A-2P) | Rel-15 |
| CA_n261(G-O) | Rel-15 |
| CA_n261(5O) | Rel-15 |
| CA_n260(O-Q) | Rel-15 |

This TR contains a band specific combination part. The actual requirements are added to the corresponding technical specifications.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c.

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BS	Base Station

CA	Carrier Aggregation

CA_nX-nY	Inter-band CA of component carrier(s) in one sub-block within Band X and component carrier(s) in one sub-block within Band Y where X and Y are the applicable NR operating band

CC	Component Carriers

DL	DownLink

FDD	Frequency Division Duplex

IMD	Inter-modulation

MSD	Maximum Sensitivity Deduction

NR	New RAT

SCS	Subcarrier spacing

TDD	Time Division Duplex

UE	User Equipment

UL	UpLink

# 4	Background

The present document is a technical report for NR Intra-band Carrier Aggregation under Rel-16 timeframe. The document covers each band combination specific issues (i.e. one sub-clause defined per band combination)

## 4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

# 5	Intra-Band Contiguous Carrier Aggregation FR1: Specific Band Combination Part

## 5.1	CA_2DL_n66B_1UL_n66A

### 5.1.1	Channel bandwidths per operating band for CA

Table 5.1.1-1: Supported bandwidth combinations for CA_n66B

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  | Aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  |  |
| CA_n66B | - | 51 | 20 | 25 | 0 |
|  |  | 10 | 15 |  |  |
|  |  | 15 | 10 |  |  |
|  |  | 20 | 51 |  |  |
|  |  | 10 | 20 | 30 |  |
|  |  | 15 | 15 |  |  |
|  |  | 20 | 10 |  |  |
|  |  | 15 | 20 | 35 |  |
|  |  | 20 | 15 |  |  |
|  |  | 51 | 40 | 45 |  |
|  |  | 40 | 51 |  |  |
|  |  | 10 | 40 | 50 |  |
|  |  | 40 | 10 |  |  |
| Note 1: 5MHz is not applicable for 30/60kHz SCS |  |  |  |  |  |

### 5.1.2	UE co-existence studies

There are no co-existence issues for this combination.

## 5.2	CA_2DL_n71B

### 5.2.1	Channel bandwidths per operating band for CA

Table 5.2.1-1: Supported bandwidth combinations for CA_n71B

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  | Aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  |  |
| CA_n71B | - | 5 | 20 | 25 | 0 |
|  |  | 10 | 15 |  |  |
|  |  | 15 | 10 |  |  |
|  |  | 20 | 5 |  |  |

### 5.2.2	UE co-existence studies

There are no co-existence issues for this combination.

### 5.2.3	REFSENS

REFSENS needs to be investigated further.

## 5.3	CA_2DL_n41C_1UL_n41A

### 5.3.1	Channel bandwidths per operating band for CA

Table 5.3.1-1: Bandwidth combination sets for Intra band contiguous CA configurations FR1

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_n41C | - | 50 | 60 |  |  |  | 110 | 0 |
|  |  | 40 | 80 |  |  |  | 120 |  |
|  |  | 60 | 60 |  |  |  |  |  |
|  |  | 50 | 80 |  |  |  | 130 |  |
|  |  | 40 | 100 |  |  |  | 140 |  |
|  |  | 60 | 80 |  |  |  |  |  |
|  |  | 50 | 100 |  |  |  | 150 |  |
|  |  | 60 | 100 |  |  |  | 160 |  |
|  |  | 80 | 80 |  |  |  |  |  |
|  |  | 80 | 100 |  |  |  | 180 |  |
|  |  | 60 | 50 |  |  |  | 110 |  |
|  |  | 80 | 40 |  |  |  | 120 |  |
|  |  | 80 | 50 |  |  |  | 130 |  |
|  |  | 100 | 40 |  |  |  | 140 |  |
|  |  | 80 | 60 |  |  |  |  |  |
|  |  | 100 | 50 |  |  |  | 150 |  |
|  |  | 100 | 60 |  |  |  | 160 |  |
|  |  | 100 | 80 |  |  |  | 180 |  |
|  |  | 10, 15, 20, 40, 50, 60, 80, 90 | 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  | 190 | 1 |

### 5.3.2	Co-existence studies

There are no co-existence issues for this combination.

## 5.4	CA_2DL_n48B, CA_2DL_n48C

### 5.4.1	Channel bandwidths per operating band for CA

Table 5.4.1-1: Supported bandwidth combinations for CA_2DL_n48B and CA_2DL_n48C

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_n48B | - | 51 | 15 |  |  |  | 20 | 0 |
|  |  | 10 | 10 |  |  |  |  |  |
|  |  | 15 | 51 |  |  |  |  |  |
|  |  | 51 | 20 |  |  |  | 25 |  |
|  |  | 10 | 15 |  |  |  |  |  |
|  |  | 15 | 10 |  |  |  |  |  |
|  |  | 20 | 51 |  |  |  |  |  |
|  |  | 10 | 20 |  |  |  | 30 |  |
|  |  | 15 | 15 |  |  |  |  |  |
|  |  | 20 | 10 |  |  |  |  |  |
|  |  | 15 | 20 |  |  |  | 35 |  |
|  |  | 20 | 15 |  |  |  |  |  |
|  |  | 20 | 20 |  |  |  | 40 |  |
|  |  | 51 | 40 |  |  |  | 45 |  |
|  |  | 40 | 51 |  |  |  |  |  |
|  |  | 10 | 40 |  |  |  | 50 |  |
|  |  | 40 | 10 |  |  |  |  |  |
| CA_n48C | - | 10 | 90 |  |  |  | 100 | 0 |
|  |  | 20 | 80 |  |  |  |  |  |
|  |  | 40 | 60 |  |  |  |  |  |
|  |  | 50 | 50 |  |  |  |  |  |
|  |  | 60 | 40 |  |  |  |  |  |
|  |  | 80 | 20 |  |  |  |  |  |
|  |  | 90 | 10 |  |  |  |  |  |
|  |  | 15 | 90 |  |  |  | 105 |  |
|  |  | 90 | 15 |  |  |  |  |  |
|  |  | 10 | 100 |  |  |  | 110 |  |
|  |  | 20 | 90 |  |  |  |  |  |
|  |  | 50 | 60 |  |  |  |  |  |
|  |  | 60 | 50 |  |  |  |  |  |
|  |  | 90 | 20 |  |  |  |  |  |
|  |  | 100 | 10 |  |  |  |  |  |
|  |  | 15 | 100 |  |  |  | 115 |  |
|  |  | 100 | 15 |  |  |  |  |  |
|  |  | 20 | 100 |  |  |  | 120 |  |
|  |  | 40 | 80 |  |  |  |  |  |
|  |  | 60 | 60 |  |  |  |  |  |
|  |  | 80 | 40 |  |  |  |  |  |
|  |  | 100 | 20 |  |  |  |  |  |
|  |  | 40 | 90 |  |  |  | 130 |  |
|  |  | 50 | 80 |  |  |  |  |  |
|  |  | 80 | 50 |  |  |  |  |  |
|  |  | 90 | 40 |  |  |  |  |  |
|  |  | 40 | 100 |  |  |  | 140 |  |
|  |  | 50 | 90 |  |  |  |  |  |
|  |  | 60 | 80 |  |  |  |  |  |
|  |  | 80 | 60 |  |  |  |  |  |
|  |  | 90 | 50 |  |  |  |  |  |
|  |  | 100 | 40 |  |  |  |  |  |
|  |  | 50 | 100 |  |  |  | 150 |  |
|  |  | 60 | 90 |  |  |  |  |  |
|  |  | 90 | 60 |  |  |  |  |  |
|  |  | 100 | 50 |  |  |  |  |  |
| NOTE 1: 51 MHz is not applicable for 30/60kHz SCS |  |  |  |  |  |  |  |  |

### 5.4.2	Co-existence studies

There are no co-existence issues for these combinations.

### 5.4.3	REFSENS

There are no REFSENS exceptions for these combinations.

## 5.5	CA_2DL_n1B_ 1UL_n1A

### 5.5.1	Operating band for CA

Table 5.5.1-1: intra-band contiguous CA operating bands in FR1

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n1 | n1 |

### 5.5.2	Channel bandwidths per operating band for CA

Table 5.5.2-1: Supported bandwidth combinations for CA_2DL_n1B_1UL _n1A

| NR CA Configuration | Uplink Configurations | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n1B | - | 10 | 10,15 | 40 | 0 |
|  |  | 15 | 15,20 |  |  |
|  |  | 20 | 20 |  |  |

### 5.5.3	Co-existence studies

There are no co-existence issues for this combination.

## 5.6	CA_2DL_n7B_ 1UL_n7A

### 5.6.1	Operating band for CA

Table 5.6.1-1: intra-band contiguous CA operating bands in FR1

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n7 | n7 |

### 5.6.2	Channel bandwidths per operating band for CA

Table 5.6.2-1: Supported bandwidth combinations for CA_2DL_n1B_1UL _n1A

| NR CA Configuration | Uplink Configurations | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n7B | - | 10, 15, 20 | 10, 15, 20, 30, 35, 40 | 50 | 0 |

### 5.6.3	Co-existence studies

There are no co-existence issues for this combination.

# 6	Intra-Band Non-Contiguous Carrier Aggregation FR1: Specific Band Combination Part

## 6.1	CA_2DL_n66(2A)_1UL_n66A

### 6.1.1	Channel bandwidths per operating band for CA

Table 6.1.1-1: Supported bandwidth combinations for CA_n66(2A)

| NR CA Configuration | Uplink Configurations | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n66(2A) | - | 51 | 20 | 25 | 0 |
|  |  | 10 | 15 |  |  |
|  |  | 20 | 51 |  |  |
|  |  | 15 | 10 |  |  |
|  |  | 10 | 20 | 30 |  |
|  |  | 15 | 15 |  |  |
|  |  | 20 | 10 |  |  |
|  |  | 15 | 20 | 35 |  |
|  |  | 20 | 15 |  |  |
|  |  | 20 | 20 | 40 |  |
|  |  | 51 | 40 | 45 |  |
|  |  | 40 | 51 |  |  |
|  |  | 10 | 40 | 50 |  |
|  |  | 40 | 10 |  |  |
|  |  | 15 | 40 | 55 |  |
|  |  | 40 | 15 |  |  |
|  |  | 20 | 40 | 60 |  |
|  |  | 40 | 20 |  |  |
| Note 1: 5MHz is not applicable for 30/60kHz SCS |  |  |  |  |  |

### 6.1.2	UE co-existence studies

There are no co-existence issues for this combination.

### 6.1.3	REFSENS

There are no REFSENS exceptions for this combination. However, UL configuration for REFSENS needs to be captured after general principles for RX requirements have been agreed.

## 6.2	CA_2DL_n41(2A)_1UL_n41A

### 6.2.1 Operating band for CA

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n41(2A) | n41 |

### 6.2.2	Channel bandwidths per operating band for CA

Table 6.2.2-1: Supported bandwidth combinations for CA_2DL_n41(2A)_1UL _n41A

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_n41(2A) | - | 40 | 40 |  |  |  | 80 | 0 |
|  |  | 40 | 50 |  |  |  | 90 |  |
|  |  | 40 | 60 |  |  |  | 100 |  |
|  |  | 50 | 50 |  |  |  |  |  |
|  |  | 50 | 60 |  |  |  | 110 |  |
|  |  | 40 | 80 |  |  |  | 120 |  |
|  |  | 60 | 60 |  |  |  |  |  |
|  |  | 50 | 80 |  |  |  | 130 |  |
|  |  | 40 | 100 |  |  |  | 140 |  |
|  |  | 60 | 80 |  |  |  |  |  |
|  |  | 50 | 100 |  |  |  | 150 |  |
|  |  | 60 | 100 |  |  |  | 160 |  |
|  |  | 80 | 80 |  |  |  |  |  |
|  |  | 50 | 40 |  |  |  | 90 |  |
|  |  | 60 | 40 |  |  |  | 100 |  |
|  |  | 60 | 50 |  |  |  | 110 |  |
|  |  | 80 | 40 |  |  |  | 120 |  |
|  |  | 80 | 50 |  |  |  | 130 |  |
|  |  | 100 | 40 |  |  |  | 140 |  |
|  |  | 80 | 60 |  |  |  |  |  |
|  |  | 100 | 50 |  |  |  | 150 |  |
|  |  | 100 | 60 |  |  |  | 160 |  |
|  |  | 100 | 80 |  |  |  | 180 |  |
|  |  | 80 | 100 |  |  |  | 180 |  |
|  |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  | 190 | 1 |

### 6.2.3	Co-existence studies

There are no co-existence issues for this combination.

### 6.2.4	REFSENS

There are no REFSENS exceptions for this combination. UL configuration for REFSENS is listed below.

Table 6.2.4-1: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity

| CA configuration | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| CA_n41(2A) | NOTE 1 | NOTE 2 | NOTE 3 | 0.0 | TDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.2-1. NOTE 2: All applicable sub-block gap sizes. NOTE 3: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE 4: The carrier center frequency of PCC in the DL operating band is configured closer to the UL operating band. |  |  |  |  |  |

## 6.3	CA_2DL_n25(2A)_1UL_n25A

### 6.3.1	Channel bandwidths per operating band for CA

Table 6.3.1-1: Supported bandwidth combinations for CA_2DL_n25(2A)_1UL _n25A

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_n25(2A) | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  |  | 40 | 0 |

### 6.3.2	Co-existence studies

There are no co-existence issues for this combination.

### 6.3.3	REFSENS

REFSENS can be impacted by the PCC UL being closer to do the SCC DL than the nominal spacing.

NOTE to rapporteur: The paragraph below has highlighted necessary changes to 38.101-1  section 7.3A.2.2 that will need to be included in the big CR.

For intra-band non-contiguous carrier aggregation with FDL_low < 2700 MHz and FUL_low < 2700 MHz with one uplink carrier and two or more downlink sub-blocks, throughput of each downlink component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) and parameters specified in Table 7.3.2-1, Table 7.3.2-2, and Table 7.3A.2.2-1 with the reference sensitivity power level increased by D RIBNC  given in Table 7.3A.2.2-1 for the SCC(s). For aggregation of more than two or more downlink FDD carriers with one uplink carrier the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3A.2.2-1. The requirements apply with all downlink carriers active. Unless given by Table 7.3.2-4, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.3-1) configured.

| CA configuration | SCS kHz | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n25(2A) | 15 | 25RB+25RB | Wgap = 55.0 | 101 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE 2: Wgap is the sub-block gap between the two sub-blocks. NOTE 3: The carrier centre frequency of SCC in the DL operating band is configured closer to the UL operating band. |  |  |  |  |  |  |

## 6.4	CA_2DL_n48(2A)_1UL_n48A

### 6.4.1	Channel bandwidths per operating band for CA

Table 6.4.1-1: Supported bandwidth combinations for CA_2DL_n48(2A)_1UL _n48A

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_n48(2A) | - | 51, 10, 15, 20, 40, 50 | 51, 10, 15, 20, 40, 50 |  |  |  | 100 | 0 |
|  |  | 10, 15, 20, 40,50, 60, 80, 90, 100 | 10, 15, 20, 40,50, 60, 80, 90, 100 |  |  |  | 1402 |  |
| NOTE 1: 5MHz is not applicable for 30/60kHz SCS NOTE 2: Parameter value accounts for both, the maximum frequency range of band n48 (150MHz), and the minimum frequency gaps in between NR non-contiguous component carriers |  |  |  |  |  |  |  |  |

### 6.4.2	Co-existence studies

There are no co-existence issues for this combination.

### 6.4.3	REFSENS

There are no REFSENS exceptions for this combination. However, UL configuration for REFSENS needs to be captured after general principles for RX requirements have been agreed

## 6.5	CA_2DL_n3(2A)_ 1UL_n3A

### 6.5.1 Operating band for CA

Table 6.5.1-1: intra-band non-contiguous CA operating bands in FR1

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n3 | n3 |

### 6.5.2 Channel bandwidths per operating band for CA

Table 6.5.2-1: Supported bandwidth combinations for CA_2DL_n3(2A)_1UL _n3A

| NR CA Configuration | Uplink Configurations | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n3(2A) | - | 5,10,15,20 | 5,10,15,20 | 40 | 0 |

### 6.5.3 Co-existence studies

### There are no co-existence issues for this combination.

### 6.5.4 REFSENS

### There are no REFSENS exceptions for this combination. UL configuration for REFSENS is listed below.

Table 6.5.4-1: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity

| CA configuration | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| CA_n3(2A) | 25RB+25RB | 45.0 < Wgap ≤ 65.0 | 126 | 4.7 | FDD |
|  |  | 0.0 < Wgap ≤ 45.0 | 256 | 0 |  |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.2-1. NOTE 2: All applicable sub-block gap sizes. NOTE 3: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE 4: The carrier center frequency of PCC in the DL operating band is configured closer to the UL operating band. NOTE 5: refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. |  |  |  |  |  |

## 6.6	CA_2DL_n7(2A)_ 1UL_n7A

### 6.6.1 Operating band for CA

Table 6.6.1-1: intra-band non-contiguous CA operating bands in FR1

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n7 | n7 |

### 6.6.2 Channel bandwidths per operating band for CA

Table 6.6.2-1: Supported bandwidth combinations for CA_2DL_n7(2A)_1UL _n7A

| NR CA Configuration | Uplink Configurations | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n7(2A) | - | 5,10,15,20 | 5,10,15,20 | 40 | 0 |

### 6.6.3 Co-existence studies

### There are no co-existence issues for this combination.

### 6.6.4 REFSENS

### There are no REFSENS exceptions for this combination. UL configuration for REFSENS is listed below.

Table 6.6.4-1: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity

| CA configuration | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| CA_n7(2A) | 52RB+25RB (SCS=15kHz) | Wgap = 55 | 325 | 0.0 | FDD |
|  |  | Wgap = 30 | 505 | 0.0 | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.2-1. NOTE 2: All applicable sub-block gap sizes. NOTE 3: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE 4: The carrier center frequency of PCC in the DL operating band is configured closer to the UL operating band. NOTE 5: Refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE 6: Wgap is the sub-block gap between the two sub-blocks. NOTE 7: The carrier centre frequency of SCC in the DL operating band is configured closer to the UL operating band. |  |  |  |  |  |

## 7.1	CA_xDL_n257a_xUL_n257a (x=2, 3, 4, 5, 6, 7, 8, a=G, H, I, J, K, L, M)

### 7.1.1	Channel bandwidths per operating band for CA

Table 7.1.1-1: NR CA configurations, bandwidth combination sets and fallback group defined for intra-band contiguous CA

|  |  | NR CA configuration / Bandwidth combination set / Fallback group |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  | Maximum aggregated BW (MHz) | BCS | Fallback group |
|  |  | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) |  |  |  |
| CA_n257G | CA_n257G | 100 | 100 |  |  |  |  |  |  | 200 | 0 | 3 |
| CA_n257H | CA_n257G CA_n257H | 100 | 100 | 100 |  |  |  |  |  | 300 | 0 |  |
| CA_n257I | CA_n257G CA_n257H CA_n257I | 100 | 100 | 100 | 100 |  |  |  |  | 400 | 0 |  |
| CA_n257J | CA_n257G CA_n257H CA_n257I CA_n257J | 100 | 100 | 100 | 100 | 100 |  |  |  | 500 | 0 |  |
| CA_n257K | CA_n257G CA_n257H CA_n257I CA_n257J CA_n257K | 100 | 100 | 100 | 100 | 100 | 100 |  |  | 600 | 0 |  |
| CA_n257L | CA_n257G CA_n257H CA_n257I CA_n257J CA_n257K CA_n257L | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  | 700 | 0 |  |
| CA_n257M | CA_n257G CA_n257H CA_n257I CA_n257J CA_n257K CA_n257L CA_n257M | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 800 | 0 |  |

### 7.1.2	UE co-existence studies

There are no co-existence issues for this combination.

## 7.2	CA_n258

### 7.2.1	Operating bands for CA

Table 7.2.1-1: Intra-band CA

| NR CA Band | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n258 | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

### 7.2.2	Channel bandwidths per operating band for CA

Table 7.2.2-1: NR CA configurations, bandwidth combination sets and fallback group defined for intra-band contiguous CA

|  |  | NR CA configuration / Bandwidth combination set / Fallback group |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  | Maximum aggregated BW (MHz) | BCS | Fallback group |
|  |  | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) |  |  |  |
| CA_n258B | CA_n258A CA_n258B | 50, 100, 200, 400 | 50, 100, 200, 400 |  |  |  |  |  |  | 800 | 0 | 1 |
| CA_n258C | CA_n258A CA_n258B CA_n258C | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 |  |  |  |  |  | 1200 | 0 |  |
| CA_n258D | CA_n258A CA_n258D | 50, 100, 200 | 50, 100, 200 |  |  |  |  |  |  | 400 | 0 | 2 |
| CA_n258E | CA_n258A CA_n258D CA_n258E | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 |  |  |  |  |  | 600 | 0 |  |
| CA_n258F | CA_n258A CA_n258D CA_n258E CA_n258F | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 |  |  |  |  | 800 | 0 |  |
| CA_n258G | CA_n258A CA_n258G | 50, 100 | 50, 100 |  |  |  |  |  |  | 200 | 0 | 3 |
| CA_n258H | CA_n258A CA_n258G CA_n258H | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  | 300 | 0 |  |
| CA_n258I | CA_n258A CA_n258G CA_n258H CA_n258I | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  | 400 | 0 |  |
| CA_n258J | CA_n258A CA_n258G CA_n258H CA_n258I CA_n258J | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  | 500 | 0 |  |
| CA_n258K | CA_n258A CA_n258G CA_n258H CA_n258I CA_n258J CA_n258K | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  | 600 | 0 |  |
| CA_n258L | CA_n258A CA_n258G CA_n258H CA_n258I CA_n258J CA_n258K CA_n258L | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  | 700 | 0 |  |
| CA_n258M | CA_n258A CA_n258G CA_n258H CA_n258I CA_n258J CA_n258K CA_n258L CA_n258M | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 800 | 0 |  |

### 7.2.3	Co-existence studies

Table 7.2.3-1: Impact of UL and DL Harmonic Interference

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  | 6th Harmonic |  | 7th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL DL Low Band Edge | UL DL High Band Edge | UL DL Low Band Edge | UL DL High Band Edge | UL DL Low Band Edge | UL DL High Band Edge | UL DL Low Band Edge | UL DL High Band Edge | UL DL Low Band Edge | UL DL High Band Edge | UL DL Low Band Edge | UL DL High Band Edge | UL DL Low Band Edge | UL DL High Band Edge |
| n258 | 24250 | 27500 | 48500 | 55000 | 72750 | 82500 | 97000 | 110000 | 121250 | 137500 | 145500 | 165000 | 169750 | 192500 |

# 8	Intra-Band Non-Contiguous Carrier Aggregation FR2: Specific Band Combination Part

## 8.1	Intra band non-contiguous CA configurations n260

Table 8.1-1: Supported bandwidth combinations for n260(A)

|  |  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |
| NR configuration | Uplink CA configurations | SCS | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Fall-back group |
| CA_n260(5A) | - | 60 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 |  |  |  | 1000 |  |
|  |  | 120 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 |  |  |  | 2000 |  |
| CA_n260(6A) | - | 60 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 |  |  | 1200 |  |
|  |  | 120 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 |  |  | 2400 |  |
| CA_n260(7A) | - | 60 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 |  | 1400 |  |
|  |  | 120 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 |  | 26001 |  |
| CA_n260(8A) | - | 60 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 1600 |  |
|  |  | 120 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 26501 |  |
| Note 1: The maximum bandwidth of band n260 is 3000MHz and a non-contiguous gap is in between NR component carriers |  |  |  |  |  |  |  |  |  |  |  |  |

Table 8.1-2: Supported bandwidth combinations for n260(D)

|  |  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |
| NR configuration | Uplink CA configurations | SCS | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Fall-back group |
| CA_n260(2D) | - | 60 | 50, 100, 200 | 200 | 50, 100, 200 | 200 |  |  |  |  | 800 | 2 |
|  |  | 120 | 50, 100, 200 | 200 | 50, 100, 200 | 200 |  |  |  |  | 800 |  |

Table 8.1-3: Supported bandwidth combinations for n260(G)

|  |  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |
| NR configuration | Uplink CA configurations | SCS | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Fall-back group |
| CA_n260(2G) | - | 60 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  | 400 | 3 |
|  |  | 120 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  | 400 |  |
| CA_n260(3G) | - | 60 | 100 | 50, 100 | 100 | 50, 100 | 100 | 50, 100 |  |  | 600 | 3 |
|  |  | 120 | 100 | 50, 100 | 100 | 50, 100 | 100 | 50, 100 |  |  | 600 |  |
| CA_n260(4G) | - | 60 | 100 | 50, 100 | 100 | 50, 100 | 100 | 50, 100 | 100 | 50, 100 | 800 | 3 |
|  |  | 120 | 100 | 50, 100 | 100 | 50, 100 | 100 | 50, 100 | 100 | 50, 100 | 800 |  |

Table 8.1-4: Supported bandwidth combinations for n260(H)

|  |  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |
| NR configuration | Uplink CA configurations | SCS | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Fall-back group |
| CA_n260(2H) | - | 60 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  | 600 | 3 |
|  |  | 120 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  | 600 |  |

Table 8.1-5: Supported bandwidth combinations for n260(O)

|  |  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |
| NR configuration | Uplink CA configurations | SCS | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Fall-back group |
| CA_n260(2O) | - | 60 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  | 400 | 4 |
|  |  | 120 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  | 400 |  |
| CA_n260(3O) | - | 60 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  | 600 | 4 |
|  |  | 120 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  | 600 |  |
| CA_n260(4O) | - | 60 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 800 | 4 |
|  |  | 120 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 800 |  |

Table 8.1-6: Supported bandwidth combinations for n260(P)

|  |  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |
| NR configuration | Uplink CA configurations | SCS | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Fall-back group |
| CA_n260(2P) | - | 60 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  | 600 | 4 |
|  |  | 120 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  | 600 |  |

Table 8.1-7: Supported bandwidth combinations for n260(P)

|  |  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | SCS | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |  |  |  |  | Aggregated BW (MHz) | Fallback group |
|  |  |  | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) |  |  |
| CA_n260(4P) | n260A | 60 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  | 1200 | 4 |
|  |  | 120 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  | 1200 |  |

Table 8.1-8: Supported bandwidth combinations for n260(Q)

|  |  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | SCS | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |  |  |  |  | Aggregated BW (MHz) | Fallback group |
|  |  |  | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) |  |  |
| CA_n260(2Q) | n260A | 60 | 50, 100 | 50, 100, | 50, 100 | 50, 100 | 50, 100 | 50, 100, | 50, 100 | 50, 100 |  |  |  |  |  |  | 800 | 4 |
|  |  | 120 | 50, 100 | 50, 100, | 50, 100 | 50, 100 | 50, 100 | 50, 100, | 50, 100 | 50, 100 |  |  |  |  |  |  | 800 |  |

## 8.2	Intra band non-contiguous CA fallback groups n260

Table 8.2-1: Supported bandwidth combinations for n260(A-G)

|  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |  |  | Maximum aggregated bandwidth (MHz) | Fall-back group |
|  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  |  |
| CA_n260(2A-G) | - | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  | See CA_n260G Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  |  | 1000 |  |
|  |  | See CA_n260G Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  |  | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |  |  |
| CA_n260(A-2G) | - | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 | See CA_n260(2G) Bandwidth Combination Fallback group 3 in table above |  |  |  |  |  |  |  |  |  |  | 800 |  |
|  |  | See CA_n260(2G) Bandwidth Combination Fallback group 3 in table above |  |  |  |  |  | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 |  |  |  |  |  |  |  |
| CA_n260(2A-2G) | - | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  | See CA_n260(2G) Bandwidth Combination Fallback group 3 in table above |  |  |  |  |  |  |  | 1200 |  |
|  |  | See CA_n260(2G) Bandwidth Combination Fallback group 3 in table above |  |  |  |  |  |  | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |
| CA_n260(2A-2G-O) | - | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  | See CA_n260(2G) Bandwidth Combination Fallback group 3 in table above |  |  |  |  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  | 1400 |  |
|  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  | See CA_n260(2G) Bandwidth Combination Fallback group 3 in table above |  |  |  |  |  |
| CA_n260(3A-2G) | - | See CA_n260(3A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  | See CA_n260(2G) Bandwidth Combination Fallback group 3 in table above |  |  |  |  |  |  | 1600 |  |
|  |  | See CA_n260(2G) Bandwidth Combination Fallback group 3 in table above |  |  |  |  |  |  |  | See CA_n260(3A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |
| CA_n260(4A-G) | - | See CA_n260(4A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  | See CA_n260G Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  |  | 1800 |  |
|  |  | See CA_n260G Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  | See CA_n260(4A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |
| CA_n260(4A-2G) | - | See CA_n260(4A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  | See CA_n260(2G) Bandwidth Combination Fallback group 3 in table above |  |  |  | 2000 |  |
|  |  | See CA_n260(2G) Bandwidth Combination Fallback group 3 in table above |  |  |  |  |  |  |  | See CA_n260(4A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |
| CA_n260(A-G) | - | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 | See CA_n260G Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  |  |  |  |  | 600 |  |
|  |  | See CA_n260G Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(3A-G) | - | See CA_n260(3A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  | See CA_n260G Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  | 1400 |  |
|  |  | See CA_n260G Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  | See CA_n260(3A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |

Table 8.2-2: Supported bandwidth combinations for n260(A-H)

|  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |  | Maximum aggregated bandwidth (MHz) | Fall-back group |
|  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  | Channel bandwidths for carrier (MHz) |  | Channel bandwidths for carrier (MHz) |  |  |
| CA_n260(A-2H) | - | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 | See CA_n260(2H) Bandwidth Combination Fallback group 3 in table above |  |  |  |  |  |  |  |  |  | 1000 |  |
|  |  | See CA_n260(2H) Bandwidth Combination Fallback group 3 in table above |  |  |  |  |  |  |  |  | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 |  |  |  |
| CA_n260(2A-H) | - | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  | See CA_n260H Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  |  |  | 1100 |  |
|  |  | See CA_n260H Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  |  | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |  |
| CA_n260(2A-2H) | - | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  | See CA_n260(2H) Bandwidth Combination Fallback group 3 in table above |  |  |  |  |  |  |  |  | 1400 |  |
|  |  | See CA_n260(2H) Bandwidth Combination Fallback group 3 in table above |  |  |  |  |  |  | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |
| CA_n260(A-H) | - | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 | See CA_n260H Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  |  |  |  | 700 |  |
|  |  | See CA_n260H Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  |  | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 |  |  |  |  |  |  |  |  |

Table 8.2-3: Supported bandwidth combinations for n260(A-O)

|  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Maximum aggregated bandwidth (MHz) | Fall-back group |
|  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  |  |  |  | Channel bandwidths for carrier (MHz) |  | Channel bandwidths for carrier (MHz) |  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  |  |
| CA_n260(2A-O) | - | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  | 1000 |  |
|  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-2O) | - | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 | See CA_n260(2O) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 |  |
|  |  | See CA_n260(2O) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  |  |  | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 |  |  |  |  |  |  |  |  |  |
| CA_n260(2G-O) | - | See CA_n260(2G) Bandwidth Combination Fallback group 3 in Table 2 above |  |  |  |  |  |  |  |  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  | 600 |  |
|  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  | See CA_n260(2G) Bandwidth Combination Fallback group 3 in Table 2 above |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-2O) | - | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  | See CA_n260(2O) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  |  |  |  |  |  |  | 1200 |  |
|  |  | See CA_n260(2O) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  |  |  |  |  | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |
| CA_n260(2A-3O) | - | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  | See CA_n260(3O) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  |  |  |  |  |  |  | 1400 |  |
|  |  | See CA_n260(3O) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  |  |  |  |  |  |  |  | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |
| CA_n260(3A-2O) | - | See CA_n260(3A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  | See CA_n260(2O) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  |  |  |  |  |  | 1600 |  |
|  |  | See CA_n260(2O) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  | See CA_n260(3A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(4A-O) | - | See CA_n260(4A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  | 1800 |  |
|  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  | See CA_n260(4A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(4A-2O) | - | See CA_n260(4A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |  |  | See CA_n260(2O) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  | 2000 |  |
|  |  | See CA_n260(2O) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  |  |  |  | See CA_n260(4A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |  |
| CA_n260(A-O) | - | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 600 |  |
|  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(G-O) | - | See CA_n260G Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  | 400 |  |
|  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  | See CA_n260G Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-G-O) | - | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 | See CA_n260G Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |  | 800 |  |
|  |  | See CA_n260G Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-G-O) | - | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  | See CA_n260G Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  | 1200 |  |
|  |  | See CA_n260G Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |  |
| CA_n260(A-2G-O) | - | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 | See CA_n260(2G) Bandwidth Combination Fallback group 3 in Table 2 above |  |  |  |  |  |  |  |  |  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  | 1000 |  |
|  |  | See CA_n260(2G) Bandwidth Combination Fallback group 3 in Table 2 above |  |  |  |  |  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  |  | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 |  |  |  |
| CA_n260(A-3O) | - | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 | See CA_n260(3O) Bandwidth Combination Fallback group 4 in Table 2 above |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 |  |
|  |  | See CA_n260(3O) Bandwidth Combination Fallback group 4 in Table 2 above |  |  |  |  |  |  |  |  |  |  |  |  | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 |  |  |  |  |  |
| CA_n260(3A-O) | - | See CA_n260(3A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |  | 1400 |  |
|  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  | See CA_n260(3A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 8.2-4: Supported bandwidth combinations for n260(A-O)

|  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |  |  |  |  | Maximum aggregated bandwidth (MHz) | Fall-back group |
|  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  | Channel bandwidths for carrier (MHz) |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  |  |
| CA_n260(A-4O) | - | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 | See CA_n260(4O) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  |  |  |  |  |  |  | 1200 |  |
|  |  | See CA_n260(4O) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  |  |  |  | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 |  |  |  |  |  |
| CA_n260(2A-4O) | - | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  | See CA_n260(4O) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  |  |  |  |  |  | 1600 |  |
|  |  | See CA_n260(4O) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  |  | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |

Table 8.2-5: Supported bandwidth combinations for n260(A-P)

|  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |  | Maximum aggregated bandwidth (MHz) | Fall-back group |
|  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  |  |
| CA_n260(A-P) | - | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 | See CA_n260P Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  |  |  |  | 700 |  |
|  |  | See CA_n260P Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-P) | - | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  | See CA_n260P Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  |  |  | 1100 |  |
|  |  | See CA_n260P Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |  |  |
| CA_n260(A-2P) | - | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 | See CA_n260(2p) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  |  |  |  | 1000 |  |
|  |  | See CA_n260(2p) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  |  | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 |  |  |  |  |
| CA_n260(2A-2P) | - | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  | See CA_n260(2p) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  |  |  | 1400 |  |
|  |  | See CA_n260(2p) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |

Table 8.2-6: Supported bandwidth combinations for n260() CA (Max #CC ≤ 8)

|  |  | NR CA configuration / set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR configuration | Uplink CA configurations | Component carriers order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Maximum aggregated bandwidth (MHz) |
|  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  | Channel bandwidths for carrier (MHz) |  |  |  |  | Channel bandwidths for carrier (MHz) |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  | Channel bandwidths for carrier (MHz) |  |  | Channel bandwidths for carrier (MHz) |  |
| CA_n260(A-D) | - | CA_n260A | See CA_n260D BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 |
|  |  | See CA_n260D BCS 0 in Table 5.5A.1-2 [2] |  |  | CA_n260A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-D) | - | CA_n260(2A) [2] |  |  | See CA_n260D BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  |  |  |  |  |  |  |  | 1200 |
|  |  | See CA_n260D BCS 0 in Table 5.5A.1-2 [2] |  |  | CA_n260(2A) [2] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-D-O) | - | CA_n260A | See CA_n260D BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  | See CA_n260O BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  |  |  | 1000 |
|  |  | See CA_n260O BCS 0 in Table 5.5A.1-2 [2] |  |  | See CA_n260D BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  | CA_n260A |  |  |  |  |  |  |  |
| CA_n260(2A-D-O) | - | CA_n260(2A) [2] |  |  | See CA_n260D BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  | See CA_n260O BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  | 1400 |
|  |  | See CA_n260O BCS 0 in Table 5.5A.1-2 [2] |  |  | See CA_n260D BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  | CA_n260(2A) [2] |  |  |  |  |  |  |  |
| CA_n260(D-2O) | - | See CA_n260D BCS 0 in Table 5.5A.1-2 [2] |  |  | See CA_n260(2O) in Table 5.5A.2-1 [2] |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 |
|  |  | See CA_n260(2O) in Table 5.5A.2-1 [2] |  |  |  |  |  |  |  |  |  | See CA_n260D BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  |  |
| CA_n260(A-D-2O) | - | CA_n260A | See CA_n260D BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  | See CA_n260(2O) in Table 5.5A.2-1 [2] |  |  |  |  |  |  |  |  | 1200 |
|  |  | See CA_n260(2O) in Table 5.5A.2-1 [2] |  |  |  |  |  |  |  |  |  | See CA_n260D BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  | CA_n260A |  |  |
| CA_n260(2A-D-2O) | - | CA_n260(2A) [2] |  |  | See CA_n260D BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  | See CA_n260(2O) in Table 5.5A.2-1 [2] |  |  |  |  |  |  | 1600 |
|  |  | See CA_n260(2O) in Table 5.5A.2-1 [2] |  |  |  |  |  |  |  |  |  | See CA_n260D BCS 0 in Table 5.5A.1-2 [2] |  |  | CA_n260(2A) [2] |  |  |  |  |
| CA_n260(A-2D) | - | CA_n260A | See CA_n260(2D) in Table 8.1-x1 above |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1200 |
|  |  | See CA_n260(2D) in Table 8.1-x1 above |  |  |  |  |  |  |  |  | CA_n260A |  |  |  |  |  |  |  |  |
| CA_n260(2A-2D) | - | CA_n260(2A) [2] |  |  | See CA_n260(2D) in Table 8.1-x1 above |  |  |  |  |  |  |  |  |  |  |  |  |  | 1600 |
|  |  | See CA_n260(2D) in Table 8.1-x1 above |  |  |  |  |  |  |  |  | CA_n260(2A) [2] |  |  |  |  |  |  |  |  |
| CA_n260(A-P) | - | CA_n260A | See CA_n260P BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 700 |
|  |  | See CA_n260P BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  | CA_n260A |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-P) | - | CA_n260(2A) [2] |  |  | See CA_n260P BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  |  |  |  |  |  |  |  | 1100 |
|  |  | See CA_n260P BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  | CA_n260(2A) [2] |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-2P) | - | CA_n260A |  | See CA_n260(2P) in Table 5.5A.2-1 [2] |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 |
|  |  | See CA_n260(2P) in Table 5.5A.2-1 [2] |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260A |  |  |  |  |
| DC_n260(2A-2P) | - | CA_n260(2A) [2] |  |  |  | See CA_n260(2P) in Table 5.5A.2-1 [2] |  |  |  |  |  |  |  |  |  |  |  |  | 1400 |
|  |  | See CA_n260(2P) in Table 5.5A.2-1 [2] |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2A) [2] |  |  |  |
| CA_n260(D-2G) | - | See CA_n260D BCS 0 in Table 5.5A.1-2 [2] |  |  | See CA_n260(2G) in Table 5.5A.2-1 [2] |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 |
|  |  | See CA_n260(2G) in Table 5.5A.2-1 [2] |  |  |  |  |  |  |  |  |  | See CA_n260D BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  |  |
| CA_n260(2D-O) | - | See CA_n260(2D) in Table 8.1-x1 above |  |  |  |  |  |  |  |  |  | See CA_n260O BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  | 1000 |
|  |  | See CA_n260O BCS 0 in Table 5.5A.1-2 [2] |  |  | See CA_n260(2D) in Table 8.1-x1 above |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(G-2O) | - | CA_n260G BCS 0 in Table 5.5A.1-2 [2] |  |  | See CA_n260(2O in Table 5.5A.2-1 [2] |  |  |  |  |  |  |  |  |  |  |  |  |  | 600 |
|  |  | See CA_n260(2O) in Table 5.5A.2-1 [2] |  |  |  |  |  |  |  |  |  | See CA_n260G BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  |  |
| CA_n260(2G-2O) | - | See CA_n260(2G) in Table 5.5A.2-1 [2] |  |  |  |  |  |  |  |  |  | See CA_n260(2O) in Table 5.5A.2-1 [2] |  |  |  |  |  |  | 800 |
|  |  | See CA_n260(2O in Table 5.5A.2-1 [2] |  |  |  |  |  |  |  |  |  | See CA_n260(2G) in Table 5.5A.2-1 [2] |  |  |  |  |  |  |  |
| CA_n260(G-3O) | - | See CA_n260G BCS 0 in Table 5.5A.1-2 [2] |  |  | See CA_n260(3O) in Table 5.5A.2-1 [2] |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 |
|  |  | See CA_n260(3O) in Table 5.5A.2-1 [2] |  |  |  |  |  |  |  |  |  |  |  |  | See CA_n260G BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |
| CA_n260(3G-O) | - | See CA_n260(3G) in Table 8.1-x1 above |  |  |  |  |  |  |  |  |  |  |  |  | See CA_n260O BCS 0 in Table 5.5A.1-2 [2] |  |  |  | 800 |
|  |  | See CA_n260O BCS 0 in Table 5.5A.1-2 [2] |  |  | See CA_n260(3G) in Table 8.1-x1 above |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(H-O) | - | See CA_n260H BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  |  | See CA_n260O BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  |  |  | 500 |
|  |  | See CA_n260O BCS 0 in Table 5.5A.1-2 [2] |  |  | See CA_n260H BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2H-O) | - | See CA_n260(2H) in Table 5.5A.2-1 [2] |  |  |  |  |  |  |  |  |  |  |  |  | See CA_n260O BCS 0 in Table 5.5A.1-2 [2] |  |  |  | 800 |
|  |  | See CA_n260O BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  | See CA_n260(2H) in Table 5.5A.2-1 [2] |  |  |  |  |  |  |  |  |  |  |  |  |

Table 8.2-7: Supported bandwidth combinations for n260() CA (Max #CC ≤ 12)

|  |  | NR CA configuration / set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR configuration | Uplink CA configurations | Component carriers order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Maximum aggregated bandwidth (MHz) |
|  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  |  |  | Channel bandwidths for carrier (MHz) |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  |
| CA_n260(3A-3O) | - | CA_n260(3A) [2] |  |  |  | See CA_n260(3O) in Table 5.5A.2-1 [2] |  |  |  |  |  |  |  |  |  |  |  |  |  | 1800 |
|  |  | See CA_n260(3O) in Table 5.5A.2-1 [2] |  |  |  |  |  |  |  |  | CA_n260(3A) [2] |  |  |  |  |  |  |  |  |  |
| CA_n260(2G-3O) | - | See CA_n260(2G) in Table 5.5A.2-1 [2] |  |  |  |  | See CA_n260(3O) in Table 5.5A.2-1 [2] |  |  |  |  |  |  |  |  |  |  |  |  | 1000 |
|  |  | See CA_n260(3O) in Table 5.5A.2-1 [2] |  |  |  |  |  |  |  |  |  | See CA_n260(2G) in Table 5.5A.2-1 [2] |  |  |  |  |  |  |  |  |
| CA_n260(G-4O) | - | See CA_n260G BCS 0 in Table 5.5A.1-2 [2] |  |  | See CA_n260(4O) in Table 5.5A.2-1 [2] |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 |
|  |  | See CA_n260(4O) in Table 5.5A.2-1 [2] |  |  |  |  |  |  |  |  |  |  | See CA_n260G BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  |  |
| CA_n260(2G-4O) | - | See CA_n260(2G) in Table 5.5A.2-1 [2] |  |  |  |  |  | See CA_n260(4O) in Table 5.5A.2-1 [2] |  |  |  |  |  |  |  |  |  |  |  | 1200 |
|  |  | See CA_n260(4O) in Table 5.5A.2-1 [2] |  |  |  |  |  |  |  |  |  |  |  |  | See CA_n260(2G) in Table 5.5A.2-1 [2] |  |  |  |  |  |
| CA_n260(4G-O) | - | See CA_n260(4G) in Table 8.1-x1 above |  |  |  |  |  |  |  |  |  |  |  |  |  | See CA_n260O BCS 0 in Table 5.5A.2-1 [2] |  |  |  | 1000 |
|  |  | See CA_n260O BCS 0 in Table 5.5A.2-1 [2] |  | See CA_n260(4G) in Table 8.1-x1 above |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 8.2-8: Supported bandwidth combinations for n260(A-O) (Max #CC≤8)

|  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] |
|  |  | CBW for carrier (MHz) | CBW for carrier (MHz) |  | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) |  |
| CA_n260(3A-O-P) | n260A CA_n260O CA_n260P | See CA_n261(3A) BCS0 in Table 5.5A.2-1 in [2] |  |  |  | See CA_n260O BCS0 in Table 5.5A.1-1 in [2] |  | See CA_n260P BCS0 in Table 5.5A.1-1 in [2] |  |  | 1700 |
|  |  | See CA_n260P BCS0 in Table 5.5A.1-1 in [2] |  |  |  | See CA_n261(3A) BCS0 in Table 5.5A.2-1 in [2] |  |  | See CA_n260O BCS0 in Table 5.5A.1-1 in [2] |  |  |
|  |  | See CA_n260O BCS0 in Table 5.5A.1-1 in [2] |  | See CA_n260P BCS0 in Table 5.5A.1-1 in [2] |  |  |  | See CA_n261(3A) BCS0 in Table 5.5A.2-1 in [2] |  |  |  |

Table 8.2-9: Supported bandwidth combinations for n260(A-P) (Max #CC≤8)

|  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] |
|  |  | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) |  |
| CA_n260(A-P-Q) | n260A CA_n260P CA_n260Q | See n260A Channel Bandwidth in Table 5.3.5-1 in [2] | See CA_n260P BCS0 in Table 5.5A.1-1 in [2] |  |  | See CA_n260Q BCS0 in Table 5.5A.1-1 in [2] |  |  |  | 1100 |
|  |  | See CA_n260Q BCS0 in Table 5.5A.1-1 in [2] |  |  |  | See n260A Channel Bandwidth in Table 5.3.5-1 in [2] | See CA_n260P BCS0 in Table 5.5A.1-1 in [2] |  |  |  |
|  |  | See CA_n260P BCS0 in Table 5.5A.1-1 in [2] |  |  | See CA_n260Q BCS0 in Table 5.5A.1-1 in [2] |  |  |  | See n260A Channel Bandwidth in Table 5.3.5-1 in [2] |  |

Table 8.2-10: Supported bandwidth combinations for n260() CA (Max #CC≤15)

|  |  | NR CA configuration / set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR configuration | Uplink CA configurations | Component carriers order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Maximum aggregated bandwidth (MHz) |
|  |  | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) |  |  | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) |  |
| CA_n260(2A-2G-2O) | n260A | See CA_n260(2A) BCS0 in Table 5.5A.2-1 in [2] |  | See CA_n260(2G) in Table 8.1-3 in [3] |  |  |  |  |  | CA_n260(2O) in Table 8.1-5 in [3] |  |  |  |  |  |  |  |  | 1600 |
|  |  | See CA_n260(2O) in Table 8.1-5 in [3] |  |  |  |  | See CA_n260(2A) BCS0 in Table 5.5A.2-1 in [2] |  |  | See CA_n260(2G) in Table 8.1-3 in [3] |  |  |  |  |  |  |  |  |  |
|  |  | See CA_n260(2G) in Table 8.1-3 in [3] |  |  |  |  | See CA_n260(2O) in Table 8.1-5 in [3] |  |  |  |  | See CA_n260(2A) BCS0 in Table 5.5A.2-1 in [2] |  |  |  |  |  |  |  |
| CA_n260(2A-4P) | n260A | See CA_n260(2A) BCS0 in Table 5.5A.2-1 in [2] |  | See CA_n260(4P) in Table 8.1-6-1 above |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2000 |
|  |  | See CA_n260(4P) in Table 8.1-6-1 above |  |  |  |  |  |  |  |  |  |  |  |  |  | See CA_n260(2A) BCS0 in Table 5.5A.2-1 in [2] |  |  |  |
| CA_n260(2A-2O-2Q) | n260A | See CA_n260(2A) BCS0 in Table 5.5A.2-1 in [2] |  | See CA_n260(2O) in Table 8.1-5 in [3] |  |  |  |  |  | See CA_n260(2Q) in Table 8.1-x-2 above |  |  |  |  |  |  |  |  | 2000 |
|  |  | See CA_n260(2Q) in Table 8.1-x-2 above |  |  |  |  |  |  |  |  |  | See CA_n260(2A) BCS0 in Table 5.5A.2-1 in [2] |  | See CA_n260(2O) in Table 8.1-5 in [3] |  |  |  |  |  |
|  |  | See CA_n260(2O) in Table 8.1-5 in [3] |  |  |  |  | See CA_n260(2Q) in Table 8.1-x-2 above |  |  |  |  |  |  |  |  | See CA_n260(2A) BCS0 in Table 5.5A.2-1 in [2] |  |  |  |
| CA_n260(4A-2Q) | n260A | See CA_n260(4A) in Table 5.5A.2-1 in [2] |  |  |  | See CA_n260(2Q) in Table 8.1-x-2 above |  |  |  |  |  |  |  |  |  |  |  |  | 2400 |
|  |  | See CA_n260(2Q) in Table 8.1-x-2 above |  |  |  |  |  |  |  |  |  | See CA_n260(4A) in Table 5.5A.2-1 in [2] |  |  |  |  |  |  |  |
| CA_n260(2A-2O-2P) | n260A | See CA_n260(2A) BCS0 in Table 5.5A.2-1 in [2] |  | See CA_n260(2O) in Table 8.1-5 in [3] |  |  |  |  |  | See CA_n260(2P) in Table 8.1-6 in [3] |  |  |  |  |  |  |  |  | 1800 |
|  |  | See CA_n260(2P) in Table 8.1-6 in [3] |  |  |  |  |  |  |  | See CA_n260(2A) BCS0 in Table 5.5A.2-1 in [2] |  | See CA_n260(2O) in Table 8.1-5 in [3] |  |  |  |  |  |  |  |
|  |  | See CA_n260(2O) in Table 8.1-5 in [3] |  |  |  |  | See CA_n260(2P) in Table 8.1-6 in [3] |  |  |  |  |  |  | See CA_n260(2A) BCS0 in Table 5.5A.2-1 in [2] |  |  |  |  |  |
| CA_n260(4A-4O) | n260A | See CA_n260(4A) in Table 5.5A.2-1 in [2] |  |  |  | See CA_n260(4O) in Table 8.1-5 in [3] |  |  |  |  |  |  |  |  |  |  |  |  | 2400 |
|  |  | See CA_n260(4O) in Table 8.1-5 in [3] |  |  |  |  |  |  |  |  |  | See CA_n260(4A) in Table 5.5A.2-1 in [2]) |  |  |  |  |  |  |  |
| CA_n260(6A-2O) | n260A | See CA_n260(6A) in Table 8.1-1 in [3] |  |  |  |  |  |  |  | See CA_n260(2O) in Table 8.1-5 in [3] |  |  |  |  |  |  |  |  | 24501 |
|  |  | See CA_n260(2O) in Table 8.1-5 in [3] |  |  |  |  | See CA_n260(6A) in Table 8.1-1 in [3] |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(6A-3O) | n260A | See CA_n260(6A) in Table 8.1-1 in [3] |  |  |  |  |  |  |  | See CA_n260(3O) in Table 8.1-5 in [3] |  |  |  |  |  |  |  |  | 26001 |
|  |  | See CA_n260(3O) in Table 8.1-5 in [3] |  |  |  |  |  |  |  | See CA_n260(6A) in Table 8.1-1 in [3] |  |  |  |  |  |  |  |  |  |
| CA_n260(6A-2P) | n260A | See CA_n260(6A) in Table 8.1-1 in [3] |  |  |  |  |  |  |  | See CA_n260(2P) in Table 8.1-6 in [3] |  |  |  |  |  |  |  |  | 26501 |
|  |  | See CA_n260(2P) in Table 8.1-6 in [3] |  |  |  |  |  |  |  | See CA_n260(6A) in Table 8.1-1 in [3] |  |  |  |  |  |  |  |  |  |
| CA_n260(8A-2O) | n260A | See CA_n260(8A) in Table 8.1-1 in [3] |  |  |  |  |  |  |  |  |  | See CA_n260(2O) in Table 8.1-5 in [3] |  |  |  |  |  |  | 25501 |
|  |  | See CA_n260(2O) in Table 8.1-5 in [3] |  |  |  |  | See CA_n260(8A) in Table 8.1-1 in [3] |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2O-2P) | n260A | See CA_n260(2O) in Table 8.1-5 in [3] |  |  |  | See CA_n260(2P) in Table 8.1-6 in [3] |  |  |  |  |  |  |  |  |  |  |  |  | 1000 |
|  |  | See CA_n260(2P) in Table 8.1-6 in [3] |  |  |  |  |  |  |  | See CA_n260(2O) in Table 8.1-5 in [3] |  |  |  |  |  |  |  |  |  |
| Note 1: The maximum bandwidth of band n260 is 3000MHz and a non-contiguous gap is in between NR component carriers |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 8.2-11: Supported bandwidth combinations for n260() CA (Max #CC≤15)

|  |  | NR CA configuration / set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR configuration | Uplink CA configurations | Component carriers order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Maximum aggregated bandwidth (MHz) |
|  |  | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) |  |  | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) |  |
| CA_n260(2A-2G-2O) | n260A | See CA_n260(2A) BCS0 in Table 5.5A.2-1 in [2] |  | See CA_n260(2G) in Table 8.1-3 in [3] |  |  |  |  |  | CA_n260(2O) in Table 8.1-5 in [3] |  |  |  |  |  |  |  |  | 1600 |
|  |  | See CA_n260(2O) in Table 8.1-5 in [3] |  |  |  |  | See CA_n260(2A) BCS0 in Table 5.5A.2-1 in [2] |  |  | See CA_n260(2G) in Table 8.1-3 in [3] |  |  |  |  |  |  |  |  |  |
|  |  | See CA_n260(2G) in Table 8.1-3 in [3] |  |  |  |  | See CA_n260(2O) in Table 8.1-5 in [3] |  |  |  |  | See CA_n260(2A) BCS0 in Table 5.5A.2-1 in [2] |  |  |  |  |  |  |  |
| CA_n260(2A-4P) | n260A | See CA_n260(2A) BCS0 in Table 5.5A.2-1 in [2] |  | See CA_n260(4P) in Table 8.1-6-1 above |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2000 |
|  |  | See CA_n260(4P) in Table 8.1-6-1 above |  |  |  |  |  |  |  |  |  |  |  |  |  | See CA_n260(2A) BCS0 in Table 5.5A.2-1 in [2] |  |  |  |
| CA_n260(2A-2O-2Q) | n260A | See CA_n260(2A) BCS0 in Table 5.5A.2-1 in [2] |  | See CA_n260(2O) in Table 8.1-5 in [3] |  |  |  |  |  | See CA_n260(2Q) in Table 8.1-x-2 above |  |  |  |  |  |  |  |  | 2000 |
|  |  | See CA_n260(2Q) in Table 8.1-x-2 above |  |  |  |  |  |  |  |  |  | See CA_n260(2A) BCS0 in Table 5.5A.2-1 in [2] |  | See CA_n260(2O) in Table 8.1-5 in [3] |  |  |  |  |  |
|  |  | See CA_n260(2O) in Table 8.1-5 in [3] |  |  |  |  | See CA_n260(2Q) in Table 8.1-x-2 above |  |  |  |  |  |  |  |  | See CA_n260(2A) BCS0 in Table 5.5A.2-1 in [2] |  |  |  |
| CA_n260(4A-2Q) | n260A | See CA_n260(4A) in Table 5.5A.2-1 in [2] |  |  |  | See CA_n260(2Q) in Table 8.1-x-2 above |  |  |  |  |  |  |  |  |  |  |  |  | 2400 |
|  |  | See CA_n260(2Q) in Table 8.1-x-2 above |  |  |  |  |  |  |  |  |  | See CA_n260(4A) in Table 5.5A.2-1 in [2] |  |  |  |  |  |  |  |
| CA_n260(2A-2O-2P) | n260A | See CA_n260(2A) BCS0 in Table 5.5A.2-1 in [2] |  | See CA_n260(2O) in Table 8.1-5 in [3] |  |  |  |  |  | See CA_n260(2P) in Table 8.1-6 in [3] |  |  |  |  |  |  |  |  | 1800 |
|  |  | See CA_n260(2P) in Table 8.1-6 in [3] |  |  |  |  |  |  |  | See CA_n260(2A) BCS0 in Table 5.5A.2-1 in [2] |  | See CA_n260(2O) in Table 8.1-5 in [3] |  |  |  |  |  |  |  |
|  |  | See CA_n260(2O) in Table 8.1-5 in [3] |  |  |  |  | See CA_n260(2P) in Table 8.1-6 in [3] |  |  |  |  |  |  | See CA_n260(2A) BCS0 in Table 5.5A.2-1 in [2] |  |  |  |  |  |
| CA_n260(4A-4O) | n260A | See CA_n260(4A) in Table 5.5A.2-1 in [2] |  |  |  | See CA_n260(4O) in Table 8.1-5 in [3] |  |  |  |  |  |  |  |  |  |  |  |  | 2400 |
|  |  | See CA_n260(4O) in Table 8.1-5 in [3] |  |  |  |  |  |  |  |  |  | See CA_n260(4A) in Table 5.5A.2-1 in [2]) |  |  |  |  |  |  |  |
| CA_n260(6A-2O) | n260A | See CA_n260(6A) in Table 8.1-1 in [3] |  |  |  |  |  |  |  | See CA_n260(2O) in Table 8.1-5 in [3] |  |  |  |  |  |  |  |  | 24501 |
|  |  | See CA_n260(2O) in Table 8.1-5 in [3] |  |  |  |  | See CA_n260(6A) in Table 8.1-1 in [3] |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(6A-3O) | n260A | See CA_n260(6A) in Table 8.1-1 in [3] |  |  |  |  |  |  |  | See CA_n260(3O) in Table 8.1-5 in [3] |  |  |  |  |  |  |  |  | 26001 |
|  |  | See CA_n260(3O) in Table 8.1-5 in [3] |  |  |  |  |  |  |  | See CA_n260(6A) in Table 8.1-1 in [3] |  |  |  |  |  |  |  |  |  |
| CA_n260(6A-2P) | n260A | See CA_n260(6A) in Table 8.1-1 in [3] |  |  |  |  |  |  |  | See CA_n260(2P) in Table 8.1-6 in [3] |  |  |  |  |  |  |  |  | 26501 |
|  |  | See CA_n260(2P) in Table 8.1-6 in [3] |  |  |  |  |  |  |  | See CA_n260(6A) in Table 8.1-1 in [3] |  |  |  |  |  |  |  |  |  |
| CA_n260(8A-2O) | n260A | See CA_n260(8A) in Table 8.1-1 in [3] |  |  |  |  |  |  |  |  |  | See CA_n260(2O) in Table 8.1-5 in [3] |  |  |  |  |  |  | 25501 |
|  |  | See CA_n260(2O) in Table 8.1-5 in [3] |  |  |  |  | See CA_n260(8A) in Table 8.1-1 in [3] |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2O-2P) | n260A | See CA_n260(2O) in Table 8.1-5 in [3] |  |  |  | See CA_n260(2P) in Table 8.1-6 in [3] |  |  |  |  |  |  |  |  |  |  |  |  | 1000 |
|  |  | See CA_n260(2P) in Table 8.1-6 in [3] |  |  |  |  |  |  |  | See CA_n260(2O) in Table 8.1-5 in [3] |  |  |  |  |  |  |  |  |  |
| Note 1: The maximum bandwidth of band n260 is 3000MHz and a non-contiguous gap is in between NR component carriers |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 8.3	Intra band non-contiguous CA configurations n261

Table 8.3-1: Supported bandwidth combinations for n261(H) and n261(I)

|  |  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |
| NR configuration | Uplink CA configurations | SCS | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Fall-back group |
| CA_n261(2H) | - | 60 | 100 | 100 | 50, 100 | 100 | 100 | 50, 100 |  |  | 600 | 3 |
|  |  | 120 | 100 | 100 | 50, 100 | 100 | 100 | 50, 100 |  |  | 600 |  |
| CA_n261(2I) | - | 60 | 100 | 100 | 100 | 50, 100 | 100 | 100 | 100 | 50, 100 | 800 | 3 |
|  |  | 120 | 100 | 100 | 100 | 50, 100 | 100 | 100 | 100 | 50, 100 | 800 |  |

Table 8.3-2: Supported bandwidth combinations for n261(D)

|  |  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | SCS | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |  |  |  |  | Aggregated BW (MHz) | Fallback group |
|  |  |  | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) |  |  |
| CA_n261(2D) | n261A | 60 | 50, 100, 200 | 200 | 50, 100, 200 | 200 |  |  |  |  |  |  |  |  |  |  | 800 | 2 |
|  |  | 120 | 200 | 50, 100, 200 | 200 | 50, 100, 200 |  |  |  |  |  |  |  |  |  |  | 800 |  |

Table 8.3-3: Supported bandwidth combinations for n261(G)

|  |  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | SCS | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |  |  |  |  | Aggregated BW (MHz) | Fallback group |
|  |  |  | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) |  |  |
| CA_n261(2G) | n261A | 60 | 100 | 50, 100 | 100 | 50, 100 |  |  |  |  |  |  |  |  |  |  | 400 | 3 |
|  |  | 120 | 50, 100 | 100 | 50, 100 | 100 |  |  |  |  |  |  |  |  |  |  | 400 |  |
| CA_n261(3G) | n261A | 60 | 100 | 50, 100 | 100 | 50, 100 | 100 | 50, 100 |  |  |  |  |  |  |  |  | 600 | 3 |
|  |  | 120 | 50, 100 | 100 | 50, 100 | 100 | 50, 100 | 100 |  |  |  |  |  |  |  |  | 600 |  |
| CA_n261(4G) | n261A | 60 | 100 | 50, 100 | 100 | 50, 100 | 100 | 50, 100 | 100 | 50, 100 |  |  |  |  |  |  | 7001 | 3 |
|  |  | 120 | 50, 100 | 100 | 50, 100 | 100 | 50, 100 | 100 | 50, 100 | 100 |  |  |  |  |  |  | 7001 |  |
| Note 1: The maximum bandwidth of band n261 is 850MHz and a non-contiguous gap is in between NR component carriers |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 8.3-x-3: Supported bandwidth combinations for n261(O)

|  |  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | SCS | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |  |  |  |  | Aggregated BW (MHz) | Fallback group |
|  |  |  | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) |  |  |
| CA_n261(2O) | n261A | 60 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  |  |  |  |  |  | 400 | 4 |
|  |  | 120 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  |  |  |  |  |  | 400 |  |
| CA_n261(4O) | n261A | 60 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  |  | 7001 | 4 |
|  |  | 120 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  |  | 7001 |  |
| CA_n261(7O) | n261A | 60 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 5501 | 4 |
|  |  | 120 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 5501 |  |
| Note 1: The maximum bandwidth of band n261 is 850MHz and a non-contiguous gap is in between NR component carriers |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 8.3-x-4: Supported bandwidth combinations for n261(P)

|  |  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | SCS | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |  |  |  |  | Aggregated BW (MHz) | Fallback group |
|  |  |  | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) |  |  |
| CA_n261(2P) | n261A | 60 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  |  |  |  | 600 | 4 |
|  |  | 120 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  |  |  |  | 600 |  |

Table 8.3-x-5: Supported bandwidth combinations for n261(Q)

|  |  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | SCS | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |  |  |  |  | Aggregated BW (MHz) | Fallback group |
|  |  |  | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) |  |  |
| CA_n261(2Q) | n261A | 60 | 50, 100 | 50, 100, | 50, 100 | 50, 100 | 50, 100 | 50, 100, | 50, 100 | 50, 100 |  |  |  |  |  |  | 800 | 4 |
|  |  | 120 | 50, 100 | 50, 100, | 50, 100 | 50, 100 | 50, 100 | 50, 100, | 50, 100 | 50, 100 |  |  |  |  |  |  | 800 |  |

## 8.4	Intra band non-contiguous CA fallback groups n261

Table 8.4-1: Supported bandwidth combinations for n261(A-H) and n261(A-I)

|  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR configuration | Uplink CA configurations (NOTE 1) | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  | Maximum aggregated bandwidth (MHz) | Fall-back group |
|  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  |  |
| CA_n261(A-H) | - | See CA_n261A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 | See CA_n261H Bandwidth Combination Fallback group 2 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  |  |  | 700 |  |
|  |  | See CA_n261H Bandwidth Combination Fallback group 2 in Table 5.5A.1-2 of 38.101-2 |  |  |  | See CA_n261A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 |  |  |  |  |  |  |  |
| CA_n261(A-I) | - | See CA_n261A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 | See CA_n261I Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  |  |  | 800 |  |
|  |  | See CA_n261I Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  | See CA_n261A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 |  |  |  |  |  |  |

Table 8.4-2: Supported bandwidth combinations for n261()

|  |  | NR CA configuration / set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR configuration | Uplink CA configurations (NOTE 1) | Component carriers order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Maximum aggregated bandwidth (MHz) |
|  |  | Channel bandwidths for carrier (MHz) |  | Channel bandwidths for carrier (MHz) |  | Channel bandwidths for carrier (MHz) |  | Channel bandwidths for carrier (MHz) |  |  | Channel bandwidths for carrier (MHz) |  |  |  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  |  | Channel bandwidths for carrier (MHz) |  |
| CA_n261(A-D) | - | CA_n261A |  | See CA_n261D BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 |
|  |  | See CA_n261D BCS 0 in Table 5.5A.1-2 [2] |  |  | CA_n261A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(A-G) | - | CA_n261A |  | See CA_n261G BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 600 |
|  |  | See CA_n261G BCS 0 in Table 5.5A.1-2 [2] |  |  |  | CA_n261A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(G-I) | - | See CA_n261G BCS 0 in Table 5.5A.1-2 [2] |  |  |  | See CA_n261I BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 600 |
|  |  | See CA_n261I BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  |  | See CA_n261G BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(H-I) | - | See CA_n261H BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  | See CA_n261I BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  |  |  |  |  |  |  | 700 |
|  |  | See CA_n261I BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  | See CA_n261H BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(G-H) | - | See CA_n261G BCS 0 in Table 5.5A.1-2 [2] |  |  |  | See CA_n261H BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 500 |
|  |  | See CA_n261H BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  | See CA_n261G BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(A-D-H) | - | CA_n261A |  | See CA_n261D BCS 0 in Table 5.5A.1-2 [2] |  |  |  | See CA_n261H BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  |  |  |  |  |  |  | 7501 |
|  |  | See CA_n261H BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  | See CA_n261D BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  | CA_n261A |  |  |  |  |  |  |  |
| CA_n261(A-G-H) | - | CA_n261A |  | See CA_n261G BCS 0 in Table 5.5A.1-2 [2] |  |  |  | See CA_n261H BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  |  |  |  |  |  |  | 7001 |
|  |  | See CA_n261H BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  | See CA_n261G BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  | CA_n261A |  |  |  |  |  |  |
| CA_n261(A-G-I) | - | CA_n261A |  | See CA_n261G BCS 0 in Table 5.5A.1-2 [2] |  |  |  | See CA_n261I BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  |  |  |  |  |  |  | 7001 |
|  |  | See CA_n261I BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  |  |  |  | See CA_n261G BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  | CA_n261A |  |  |  |
| CA_n261(A-H-I) | - | CA_n261A |  | See CA_n261H BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  |  |  | See CA_n261I BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  |  | 7501 |
|  |  | See CA_n261I BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  |  |  |  |  |  | See CA_n261H BCS 0 in Table 5.5A.1-2 [2] |  |  |  |  |  | CA_n261A |  |  |
| CA_n261(A-2H) |  | CA_n261A | See CA_n261(2H) in Table 5.5A.1-2 [2] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7001 |
|  |  | See CA_n261(2H) in Table 5.5A.1-2 [2] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n261A |  |  |  |
| Note 1: The maximum bandwidth of band n261 is 850MHz and a non-contiguous gap is between NR component carriers |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 8.4-3: Supported bandwidth combinations for n26() CA (Max #CC ≤ 12)

|  |  | NR CA configuration / set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR configuration | Uplink CA configurations | Component carriers order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Maximum aggregated bandwidth (MHz) |  |
|  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  | Channel bandwidths for carrier (MHz) |  | Channel bandwidths for carrier (MHz) |  | Channel bandwidths for carrier (MHz) |  |  |
| CA_n261(A-2I) | - | CA_n261A | See CA_n261(2I) in Table 8.3-x1 above |  |  |  |  |  |  |  |  |  |  |  |  |  | 7501 |  |
|  |  | See CA_n261(2I) in Table 8.3-x1 above |  |  |  |  |  |  |  | CA_n261A |  |  |  |  |  |  |  |  |
| Note 1: The maximum bandwidth of band n261 is 850MHz and a non-contiguous gap is between NR component carriers |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 8.4-4: Supported bandwidth combinations for n261() CA (Max #CC≤ 15)

|  |  | NR CA configuration / set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR configuration | Uplink CA configurations | Component carriers order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Maximum aggregated bandwidth (MHz) |
|  |  | CBW for carrier (MHz) | CBW for carrier (MHz) |  | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) | CBW for carrier (MHz) |  |
| CA_n261(A-2D) | n261A | See n261A in Table 5.3A.4-1 in [2] |  | See CA_n261(2D) in Table 8.3-x-1 above |  |  |  |  |  |  |  |  |  |  |  |  |  | 7501 |
|  |  | See CA_n261(2D) in Table 8.3-x-1 above |  |  |  |  | See n261A in Table 5.3A.4-1 in [2] |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(A-2G-2O) | n261A | See n261A in Table 5.3A.4-1 in [2] |  | CA_n261(2G) in Table 8.3-x-2 above |  |  |  | See CA_n261(2O) in Table 8.3-x-3 above |  |  |  |  |  |  |  |  |  | 6501 |
|  |  | See CA_n261(2O) in Table 8.3-x-3 above |  |  |  |  | See n261A in Table 5.3A.4-1 in [2] | CA_n261(2G) in Table 8.3-x-2 above |  |  |  |  |  |  |  |  |  |  |
|  |  | CA_n261(2G) in Table 8.3-x-2 above |  |  |  |  | See CA_n261(2O) in Table 8.3-x-3 above |  |  |  | See n261A in Table 5.3A.4-1 in [2] |  |  |  |  |  |  |  |
| CA_n261(A-3G-O) | n261A | See n261A in Table 5.3A.4-1 in [2] |  | See CA_n261(3G) in Table 8.3-x-2 above |  |  |  |  |  | See CA_n261O in Table 5.5A.1-1 in [2] |  |  |  |  |  |  |  | 6501 |
|  |  | See CA_n261O in Table 5.5A.1-1 in [2] |  |  | See n261A in Table 5.3A.4-1 in [2] | See CA_n261(3G) in Table 8.3-x-2 above |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | See CA_n261(3G) in Table 8.3-x-2 above |  |  |  |  |  |  | See CA_n261O in Table 5.5A.1-1 in [2] |  | See n261A in Table 5.3A.4-1 in [2] |  |  |  |  |  |  |  |
| CA_n261(A-4G) | n261A | See n261A in Table 5.3A.4-1 in [2] |  | See CA_n261(4G) in Table 8.3-x-2 above |  |  |  |  |  |  |  |  |  |  |  |  |  | 6501 |
|  |  | See CA_n261(4G) in Table 8.3-x-2 above |  |  |  |  |  |  |  |  | See n261A in Table 5.3A.4-1 in [2] |  |  |  |  |  |  |  |
| CA_n261(A-4O) | n261A | See n261A in Table 5.3A.4-1 in [2] |  | See CA_n261(4O) in Table 8.3-x-3 above |  |  |  |  |  |  |  |  |  |  |  |  |  | 6501 |
|  |  | See CA_n261(4O) in Table 8.3-x-3 above |  |  |  |  |  |  |  |  | See n261A in Table 5.3A.4-1 in [2] |  |  |  |  |  |  |  |
| CA_n261(A-7O) | n261A | See n261A in Table 5.3A.4-1 in [2] |  | See CA_n261(7O) in Table 8.3-x-3 above |  |  |  |  |  |  |  |  |  |  |  |  |  | 5001 |
|  |  | See CA_n261(7O) in Table 8.3-x-3 above |  |  |  |  |  |  |  |  |  |  |  |  |  |  | See n261A in Table 5.3A.4-1 in [2] |  |
| CA_n261(A-2P) | n261A | See n261A in Table 5.3A.4-1 in [2] |  | See CA_n261(2P) in Table 8.3-x-4 above |  |  |  |  |  |  |  |  |  |  |  |  |  | 7501 |
|  |  | See CA_n261(2P) in Table 8.3-x-4 above |  |  |  |  |  |  | See n261A in Table 5.3A.4-1 in [2] |  |  |  |  |  |  |  |  |  |
| CA_n261(A-2Q) | n261A | See n261A in Table 5.3A.4-1 in [2] |  | See CA_n261(2Q) in Table 8.3-x-5 above |  |  |  |  |  |  |  |  |  |  |  |  |  | 7501 |
|  |  | See CA_n261(2Q) in Table 8.3-x-5 above |  |  |  |  |  |  |  |  | See n261A in Table 5.3A.4-1 in [2] |  |  |  |  |  |  |  |
| CA_n261(A-D-2O) | n261A | See n261A in Table 5.3A.4-1 in [2] |  | See CA_n261D in Table 5.5A.1-2 in [2] |  | See CA_n261(2O) in Table 8.3-x-3 above |  |  |  |  |  |  |  |  |  |  |  | 7001 |
|  |  | See CA_n261(2O) in Table 8.3-x-3 above |  |  |  |  | See n261A in Table 5.3A.4-1 in [2] | See CA_n261D in Table 5.5A.1-2 in [2] |  |  |  |  |  |  |  |  |  |  |
|  |  | See CA_n261D in Table 5.5A.1-2 in [2] |  |  | See CA_n261(2O) in Table 8.3-x-3 above |  |  |  | See n261A in Table 5.3A.4-1 in [2] |  |  |  |  |  |  |  |  |  |
| Note 1: The maximum bandwidth of band n261 is 850MHz and a non-contiguous gap is in between NR component carriers |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Annex A: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2018-08 | 3GPP RAN4 #88 | R4-1810381 |  |  | Initial TR skeleton |  | 0.0.1 |
| 2018-10 | 3GPP RAN4 #88bis | R4-1812779 |  |  | Implemented TP´s from RAN4 #88: R4-1811433, “TP for TR38.716-01-01: Requirements for CA_n66(2A) and CA_n66B”, Dish Network R4-1811441, “TP for TR 38.716-01-01 NR Intra-band n260 CA”, Verizon UK Ltd R4-1811442, “TP for TR 38.716-01-01 NR Intra-band n261 CA”, Verizon UK Ltd | 0.0.1 | 0.1.0 |
| 2018-11 | 3GPP RAN4 #89 | R4-1815794 |  |  | Implemented TP´s from RAN4 #88bis: R4-1812078, “draft CR to introduce BCS for CA_n71B”, T-Mobile USA Inc. R4-1813789, “TP for TR 38.716-01-01 for CA_n71B”, Ericsson, T-Mobile US R4-1812347. “TP for TR 38.716-01-01 NR Intra-band n260 and n261 CA”, Verizon UK Ltd | 0.1.0 | 0.2.0 |
| 2019-02 | 3GPP RAN4 #90 | R4-1901416 |  |  | Implemented TP´s from RAN4 #89: R4-1814927, “TP for TR 38.716-01-01 for CA_2DL_n41C_1UL_n41A”, Huawei, HiSilicon R4-1816172, “TP for TR 38.716-01-01 for CA_2DL_n41(2A)_1UL_n41A”, Huawei, HiSilicon R4-1815066, “TP for TR 37.716-01-01 CA_n257_UL_n257”, NTT DOCOMO, INC. R4-1815821, “TP for 38 716-01-01 for Intra-band CA_n258B - CA_n258M”, Ericsson, Telstra | 0.2.0 | 0.3.0 |
| 2019-04 | 3GPP RAN4 #90bis | R4-1904402 |  |  | Implemented TP´s from RAN4 #90: R4-1901419, “TP for 38.716-01-01 for updated scope from RAN #82”, Ericsson R4-1901422, “TP for TR 38.716-01-01 for symbols and abbreviations”, Ericsson R4-1902123, “NR Intra-band non-contiguous CA n260 and n261”, Verizon, Ericsson | 0.3.0 | 0.4.0 |
| 2019-05 | 3GPP RAN4 #91 | R4-1906734 |  |  | Implemented TP´s from RAN4 #90bis: R4-1904404, “TP for 38.716-01-01 for updated scope from RAN #83”, Ericsson R4-1903187, “Updated TP for TR 38.716-01-01 for DL_n41(2A)_UL_n41A”, Huawei, HiSilicon R4-1904540, “TP for TR 38.716-01-01: CA_n25(2A)”, Sprint Corporation | 0.4.0 | 0.5.0 |
| 2019-08 | 3GPP RAN4 #92 | R4-1909784 |  |  | Implemented TP´s from RAN4 #91: R4-1907464, “TP for TR 38.716-01-01: CA_n48B and CA_n48C”, Samsung R4-1905414, “TP for TR 38.716-01-01: CA_n48(2A)”, Samsung | 0.5.0 | 0.6.0 |
| 2019-10 | 3GPP RAN4 #92bis | R4-1912234 |  |  | Implemented TP´s from RAN4 #92: R4-1910204, “TP for TR 38.716-01-01 for updated scope from RAN #84”, Ericsson R4-1910298, “TP for 38.716-01-01 CA_n25(2A) REFSENS”, Sprint Corporation R4-1908935, “TP for TR 38.716-01-01: DL_n1B_UL_n1A”, Huawei, HiSilicon R4-1909895, “TP for 38.716-01-01: CA_n41C and CA_n41(2A) BCS1”, Sprint Corporation | 0.6.0 | 0.7.0 |
| 2019-11 | 3GPP RAN4 #93 | R4-1914682 |  |  | Implemented TP´s from RAN4 #92bis: R4-1912236, “TP for TR 38.716-01-01 for updated scope from RAN #85”, Ericsson R4-1911471, “TP for TR 38.716-01-01: DL_n3(2A)_UL_n3A”, Huawei, HiSilicon R4-1912612, “TP for TR 38.716-01-01: DL_n7(2A)_UL_n7A”, Huawei, HiSilicon R4-1912278, “TP for TR 38.716-01-01 to include CA_n7B”, Ericsson, Telstra R4-1912564, “TP for TR 38 716-01-01 to include UL n258B - n258M”, Ericsson, Telstra | 0.7.0 | 0.8.0 |