| 3GPP TR 38.809 V0.2.0(2020-06) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Background for Integrated access and backhaul radio transmission and reception (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners’ Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	General aspect	8

4.1	Relation with other core specification	8

4.2	RF Requirements reference points	8

4.3	IAB classification	9

4.4	Regulatory aspects	9

4.5	IAB architecture	9

5   Operating band and channel arrangement	11

5.1	General	11

5.2	Operating bands	11

5.3	Channel bandwidth	11

5.4	Channel arrangement	12

6   Coexistence study	12

6.1	System layout and scenario	12

6.1.x	Co-location	12

6.2	Simulation assumption	13

6.2.x	Antenna configuration	13

6.2.x.1	General	13

6.2.x.2	FR1	13

6.2.x.3                  FR2	15

6.3	Simulation result	17

6.4	Summary on co-existence study	17

7	Conducted transmitter characteristics	17

7.1	General	17

7.2	IAB output power	17

7.3	Output power dynamics	17

7.4	Transmit ON/OFF power	17

7.5	Transmitted signal quality	18

7.5.1         IAB-DU transmitted signal quality	18

7.6	Unwanted emissions	18

7.7	Transmitter intermodulation	18

8	Conducted receiver characteristics	18

8.1	General	18

8.2	Reference sensitivity level	18

8.3	Dynamic range	18

8.4	In-band selectivity and blocking	18

8.5	Out-of-band blocking	18

8.6	Receiver spurious emissions	18

8.7	Receiver intermodulation	18

8.8	In-channel selectivity	19

9	Radiated transmitter characteristics	19

9.1	General	19

9.2	Radiated transmit power	19

9.3	IAB OTA output power	19

9.4	OTA output power dynamics	19

9.5	OTA transmit ON/OFF power	19

9.6	OTA transmitted signal quality	19

9.6.1 IAB-DU OTA transmitted signal quality	19

9.7	OTA unwanted emissions	19

9.8	OTA transmitter intermodulation	19

9.8.1 IAB DU intermodulation	20

9.8.2 IAB MT intermodulation	20

9.9	Beam correspondence for IAB-MT	20

10	Radiated receiver characteristics	20

10.1	General	20

10.2	OTA sensitivity	20

10.3	OTA reference sensitivity level	20

10.4	OTA Dynamic range	21

10.5	OTA in-band selectivity and blocking	21

10.6	OTA out-of-band blocking	21

10.7	OTA receiver spurious emissions	21

10.8	OTA receiver intermodulation	21

10.9	OTA in-channel selectivity	21

11	IAB RRM requirements	21

12	IAB EMC requirements	21

12.1    IAB EMC Emission requirements	21

12.3    IAB EMC immunity requirements	22

12.3.1 Radiated Immunity requirements	22

12.3.1.1 One enclosure case	22

12.3.1.2 Different enclosure case	22

12.3.1.4 Summary	22

12.3.2  Other Immunity requirements	22

13	IAB performance requirements	22

14	IAB conformance testing aspect	22

Annex <A> (normative): <Normative annex for a Technical Report>	23

Annex <X> (informative): Change history	24

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

Y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

Should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

Can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

Will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document is the Technical Report for the Work Item on Integrated Access and Backhaul for NR. The present document captures the background and the decisions on IAB requirements.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

Example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

4	General aspect

4.1	Relation with other core specification

IAB (Integrated Access and backhaul) node is a RAN node that supports wireless access to Ues and wirelessly backhauls the access traffic. Direct application of only Base station RF requirements is not appropriate enabler for both the wireless backhaul and wireless access and thus a new TS spec 38.174 will be specified.

In R16 IAB WI, the wireless backhaul technology is based on NR Uu so the 3GPP terminology will be reused to define the wireless backhaul RF specification. In Figure 6.3.1-1 reference diagram of architecture in 38.874, IAB node is functional splitted with IAB-DU and IAB-MT. IAB-DU has interface with UE and IAB-MT so it is representing the Base Station functionality. IAB-MT has interface with IAB-DU only and thus is representing the UE functionality.

3GPP TS 38.174 is a Single RAT NR IAB specification. The single RAT IAB means the same RAT will apply to both IAB-DU and IAB-MT. It is expected to capture IAB-DU and IAB-MT requirements for the following aspects:

Tx, Rx and demodulation core requirements for NR IAB-DU and NR IAB-MT

-	Conducted and radiated sets of core requirements for the above listed categories (i.e. Tx, Rx and IAB demodulation),

-	Requirements for FR1 and FR2 frequency ranges

-	FR1: Both conducted and OTA requirements will be required for Range 1. The applicability may depend on the requirements.

-		Requirement set 1-H: Conducted requirements and OTA requirements for FR1 hybrid IAB-DU and IAB-MT (which includes antenna functionality).

-	Requirement set 1-O: OTA requirements for FR1 OTA IAB-DU and OTA IAB-MT (which includes antenna functionality).

-	FR2: Only OTA requirements are defined for FR2.

-	Requirement set 2-O: OTA requirements for FR2 OTA IAB-DU and OTA IAB-MT.

3GPP TS 38.104 [3] is a Single RAT NR BS specification. IAB-DU shall reuse the relevant requirements from spec in TS 38.104. Where applicable, the IAB-MT may also re-use requirements from 38.104.

3GPP TS 38.101-1/2 is a Single RAT NR UE specification. Where applicable, IAB-MT may reuse requirements from the spec in TS 38.101-1/2.

4.2	RF Requirements reference points

The requirement reference points for BS is defined as below:

Table 4.2-1: BS requirement sets

| BS type / Requirement set | BS Description | Additional information |
| --- | --- | --- |
| 1-H | A BS operating at FR1 with a requirement set holding requirements defined at the TAB and OTA requirements defined at RIB. | The requirement set is like the one defined for Hybrid AAS BS. Following the approach used in 3GPP TS 37.105 |
| 1-O | A BS operating at FR1 with a requirement set consisting only OTA requirements defined at the RIB. | Following the approach developed in Eaas and documented in 3GPP TR 37.843. |
| 2-O | A BS operating at FR2 with a requirement set consisting only of OTA requirements defined at the RIB. | This requirement set is relevant for AAS BS and does not require access to RF connectors. |

The test and reference point of RF requirement for BS should be reused and thus the test spec will not be impacted.

The IAB-Du and IAB_MT may share the same hardware implementation, of so the reference points for the IAB-DU and the IAB-MT may be different.

Table 4.2-2: IAB requirement sets

| IAB type / Requirement set | IAB Description | Additional information |
| --- | --- | --- |
| 1-H | IAB-MT and IAB-DU operating at FR1 with a requirement set holding requirements defined at the respective TAB and OTA requirements defined at the respective RIB | The requirement set for both IAB-DU and IAB-MT. |
| 1-O | IAB-MT and IAB-DU operating at FR1 with a requirement set consisting only of OTA requirements defined at the respective RIB. | The requirement set for both IAB-DU and IAB-MT. |
| 2-O | IAB-MT and IAB-DU operating at FR2 with a requirement set consisting only of OTA requirements defined at the respective RIB. | The requirement set for both IAB-DU and IAB-MT. |

4.3	IAB classification

IAB-MT classification and IAB-DU classification will be defined separately.

For IAB-DU the classification will be the same as NR BS classification in TS 38.104 [x] to support Wide Area IAB-DU, Medium Range IAB-DU and Local Area IAB-DU with the same deployment scenarios for each class as for BS. The same criteria and exactly the same parameter of NR BS classification, e,g, BS to UE minimum distance on the ground for BS without connector, IAB-DU to UE minimum coupling loss for BS with connector, will be applied for IAB-DU with modification as IAB-DU to UE minimum distance on the ground for IAB-DU type 2-O/1-O and IAB-DU minimum coupling loss depends on IAB-DU type 1-H.

For IAB-MT multiple classes will be defined based on deployment scenario as well.

4.4	Regulatory aspects

<Text will be added>

4.5	IAB architecture

The logical architecture (considering the RF interfaces) of the IAB is as follows:

Figure 4.5-1: Logical IAB functions showing RF interfaces

In the RF specification these logical functions have been separated into the HW entities the IAB-DU and the IAB-MT IAB-DU. The HW entities may be implemented in the same radio hardware as shown in figure 4.2-2 or separate radio hardware as shown in figure 4.5-3, the diagrams show the OTA architecture but the same applies for the hybrid architecture.

Figure 4.5-2: Shared IAB hardware

Figure 4.5-3: Separate IAB hardware

The figures show the hardware as either completely shared or completely separate, there are of course many levels of possible integration in between these 2 extremes.

When considering the OTA architecture and specifications the nature of HW is not relevant as the node can be treated as a black box, however in deriving the requirements for the IAB nodes it is important to consider both implementations so that both can be implemented if required.

5   Operating band and channel arrangement

5.1	General

The Frequency range definitions for IAB will be the same as for the NR BS in TS 38.104 [x].

5.2	Operating bands

Operating bands will be added to the IAB specification as they are approved, no bands are precluded at this stage and can be added based on consensus. Currently the FR1 bands shown in table 5.2-1 and all Rel-15 FR2 bands shown in table 5.2-2 are to be included in the IAB specification.

Table 5.2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive FUL,low – FUL,high FDL,low – FDL,high | Duplex mode |
| --- | --- | --- |
| n41 | 2496 MHz – 2690 MHz | TDD |
| n77 | 3300 MHz – 4200 MHz | TDD |
| n78 | 3300 MHz – 3800 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | TDD |

Table 5.2-2: NR operating bands in FR2

| NR operating band | Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive FUL,low – FUL,high FDL,low – FDL,high | Duplex mode |
| --- | --- | --- |
| n257 | 26500 MHz – 29500 MHz | TDD |
| n258 | 24250 MHz – 27500 MHz | TDD |
| n260 | 37000 MHz – 40000 MHz | TDD |
| n261 | 27500 MHz – 28350 MHz | TDD |

5.3	Channel bandwidth

Channel bandwidth covers definitions on the transmission bandwidth configuration, minimum guard band, RB alignment and channel bandwidth per operating band. For the transmission bandwidth configuration and minimum guard band, existing NR definition for BS and UE for each frequency range is the same. And they should be applied for IAB as it is. For BR alignment the IAB-DU should follow NR BS definition, And IAB-MT should follow NR UE definition. Regarding Channel bandwidth per operating band, it is agreed to incorporate only the bands and associated channel bandwidth to be supported for IAB in IAB specification.

5.4	Channel arrangement

Channel arrangement includes channel spacing, channel raster and sync raster. The introduction of IAB node should be transparent to UE from IAB-DU perspective. And it is not expected to impact on existing fundamental NR design on global channel raster and sync raster as well. Hence the channel raster, channel raster and sync raster definition in existing NR specification, which is identical in BS and UE, will be applied for IAB.

6   Coexistence study

6.1	System layout and scenario

<Text will be added>

6.1.x	Co-location

An IAB node is capable of transmitting in the DL (IAB-DU) or the UL (IAB-MT). When acting as an IAB-MT there are 2 possible co-location interference scenarios between the IAB-MT and a BS.

- Aggressor IAB-MT transmitting in UL, victim BS receiving in UL

- Aggressor BS transmitting in DL, victim IAB-MT receiving in DL

For co-location, the interference is given by:

Pinterference =10*log1010PACLR10+10PACS10

Where:

PACLR = Ptx_aggressor – ACLRaggressor – coupling

PACS = Ptx_aggressor – ACSvictim – coupling

A conservative estimate for the coupling between two co-located systems is; 30Db for FR1 and 45Db for FR2.

Note: this figure is used only for this analysis. It is not an agreed FR2 isolation figure.

For a micro BS scenario:

Table 6.1.x-1: Co-location interference between BS and IAB-MT for FR1 and FR2

|  |  | IAB |  | BS |  |
| --- | --- | --- | --- | --- | --- |
|  | unit | FR1 | FR2 | FR1 | FR2 |
| Ptx | dBm | 30 | 30 | 33 | 33 |
| ACLR | Db | 45 (Note1) | 28 (Note1) | 45 | 28 |
| Sensitivity (FR2 approx. equivalent conducted sensitivity) | dBm | -96.5 (4.5MHz) (Note2) | approx. -85 (50MHz) (Note2) | -96.5 (4.5MHz) | approx. -85 (50MHz) |
| ACS | Db | 45 | 24 | 45 | 24 |
| Coupling | Db | 30 | 45 (Note3) | 30 | 45 (Note3) |
| IAB to BS interference (UL) | dBm |  |  | -42.0 | -37.5 |
| BS to IAB interference (DL) | dBm | -41.9 | -34.5 |  |  |
| Note1: the ACLR figures used are BS values, it has not been agreed to use BS figure for IAB, however UE figures will result in worse interference. Note 2: sensitivity values based on NF assumption in co-location simulation see sub-clause x.x.x Note 3: coupling figures for FR2 are not formally agreed, assumption used only for this example |  |  |  |  |  |

Note for FR2 there are no conducted requirements so the coupling and the sensitivity are estimated to a virtual conducted point for the purposes of comparison.

It can be seen that for both FR1 and FR2 significant additional isolation (50 to 60Db) is required if the systems are to be co-located.

The issue exists for both scenario 1 and scenario 2 (see sub-clause 6.1.y) as it occurs in both the UL and the DL.

6.2	Simulation assumption

Detailed structure of the subclause is TBD.

6.2.x	Antenna configuration

6.2.x.1	General

Since some parameters required by the array antenna model are not independent, arbitrary parameter values are not supported. If parameters are selected arbitrary the model will produce incorrect gain characteristics.

We define arrays using the number of columns, rows, the separation between them as well as the definition of the element radiation pattern and its gain.

Clearly the element cannot be physically larger than the space between the elements. The element beam width parameters are directly related to the available unit area for the element. Also, the element gain is directly related to the element directivity via the selected beam widths. Therefore, parameters for GEmax, 3Db and 3Db cannot be selected arbitrary.

The array spacing, the element gain and the element beam width must therefore all be aligned.

The element peak gain will be determined by the available physical area as:

GE,max≤10log104πAλ2-LE

, where A is the area available for a single element. The area can be expressed as:

A=dhdv

Also, the maximum achieved peak element gain for given wide symmetrical beam with can be expressed as [3]:

GE,max≈10log1052525φ3dBθ3dB-LE

6.2.x.2	FR1

The FR1 antenna is defined as:

Table 6.2.x.2-1 FR1 IAB antenna model for macro scenario

| Parameter | Values |
| --- | --- |
| Composite Array radiation pattern in Db | AAθ,φ=AE"θ,φ+10log101+ρ*n=1Nm=1Mwn,m*vn,m2-1 the steering matrix components are given by vn,m=expi*2πm-1*dMλcosθ+n-1*dNλsinθsinφ n=1,2,…N, m=1,2,…M the weighting factor is given by wn,m=1NMexp1*2πm-1dMλsinθ-n-1dNλcosθsinφ n=1,2,…N, m=1,2,…M |
| Antenna element vertical radiation pattern (Db) | AE,Vθ"=-min12θ"-90°θ3dB2,SLAV,θ3dB=65°,SLAV=30dB |
| Antenna element horizontal radiation pattern (Db) | AE,Hφ"=-min12φ"φ3dB2,Am,φ3dB=130°,Am=30dB |
| Combining method for 3D antenna element pattern (Db) | AE''θ'',φ''=-min-AE,Vθ''+AE,Hφ'',Am |
| Maximum directional gain of an antenna element GE,max | 5 dBi |
| Antenna loss /Efficiency | 1.8 Db |
| BS antenna configuration | (Mg, Ng, M, N, P) = (1, 1, 8, 8, 1) Note 1,2 |
| (dv, dh) | (0.8λ, 0.5λ) |
| Mechanical down tilt | 10° |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: single polarization simulated under the assumption of polarization match. |  |

The element spacing is and hence the maximum element size is 0.8λ, 0.5λ, this corresponds to an element gain or approx.:

GANT_element≈10*log104π*dv,*dhλ2-Loss≈10*log104π*0.8λ,*0.5λλ2-1.8≈5dBi

The radiation pattern for the 0.8λ, 0.5λ element has a beam width of approx. 65° in elevation and 130° in azimuth.

6.2.x.3 FR2

The FR2 BS antenna is defined as:

Table 6.2.x.3-1. FR2 IAB antenna model for macro scenario

| Parameter | Values |
| --- | --- |
| Composite Array radiation pattern in Db | AAθ,φ=AE"θ,φ+10log101+ρ*n=1Nm=1Mwn,m*vn,m2-1 the steering matrix components are given by vn,m=expi*2πm-1*dMλcosθ+n-1*dNλsinθsinφ n=1,2,…N, m=1,2,…M the weighting factor is given by wn,m=1NMexp1*2πm-1dMλsinθ-n-1dNλcosθsinφ n=1,2,…N, m=1,2,…M |
| Antenna element vertical radiation pattern (Db) | AE,Vθ"=-min12θ"-90°θ3dB2,SLAV,θ3dB=130°,SLAV=30dB |
| Antenna element horizontal radiation pattern (Db) | AE,Hφ"=-min12φ"φ3dB2,Am,φ3dB=130°,Am=30dB |
| Combining method for 3D antenna element pattern (Db) | A''θ'',φ''=-min-AE,Vθ''+AE,Hφ'',Am |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 1.8Db loss) |
| Antenna loss /Efficiency | 1.8 Db |
| BS antenna configuration | (Mg, Ng, M, N, P) = (1, 1, 8, 16, 1) Note 1,2 |
| (dv, dh) | (0.5λ, 0.5λ) |
| Mechanical down tilt | 10° |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: single polarization simulated under the assumption of polarization match. |  |

In this case the element spacing is and hence the maximum element size is 0.5λ, 0.5λ, this corresponds to an element gain or approx.:

GANT_element≈10*log104π*dv,*dhλ2-Loss≈10*log104π*0.5λ,*0.5λλ2-1.8≈3dBi

The radiation pattern for the 0.5λ, 0.5λ element has a beam width of approx. 130° in elevation and 130° in azimuth.

The UE antenna is defined as:

Table 6.2.x.3-2. FR2 UE antenna model

| Parameter | Values |
| --- | --- |
| Composite Array radiation pattern in Db | AAθ,φ=AE"θ,φ+10log101+ρ*n=1Nm=1Mwn,m*vn,m2-1 the steering matrix components are given by vn,m=expi*2πm-1*dMλcosθ+n-1*dNλsinθsinφ n=1,2,…N, m=1,2,…M the weighting factor is given by wn,m=1NMexp1*2πm-1dMλsinθ-n-1dNλcosθsinφ n=1,2,…N, m=1,2,…M |
| Antenna element vertical radiation pattern (Db) | AE,Vθ"=-min12θ"-90°θ3dB2,SLAV,θ3dB=130°,SLAV=25dB |
| Antenna element horizontal radiation pattern (Db) | AE,Hφ"=-min12φ"φ3dB2,Am,φ3dB=130°,Am=25dB |
| Combining method for 3D antenna element pattern (Db) | A''θ'',φ''=-min-AE,Vθ''+AE,Hφ'',Am |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 1.8Db loss) |
| Antenna loss /Efficiency | 1.8 Db |
| UE antenna configuration | (Mg, Ng, M, N, P) = (1, 1, 2, 2, 1) |
| (dv, dh) | (0.5λ, 0.5λ) |
| UE orientation | Random orientation in the azimuth domain: uniformly distributed between -90 and 90 degrees* Fixed elevation: 90 degrees |
| NOTE: This is done to emulate two panels: the configuration is equivalent to 2 panels with 180 shift in horizontal orientation and UE orientation uniformly distributed in the azimuth domain between -180 and 180 degrees. |  |

The element definition is the same as that of the BS but the array is smaller.

By combining the element and array patterns this gives a composite gain of:

GANT_composite≈10*log104π*dv,*dhλ2-Loss≈10*log10M*N+GANTelement6+3≈9dBi

6.3	Simulation result

<Text will be added>

6.4	Summary on co-existence study

<Text will be added>

7	Conducted transmitter characteristics

7.1	General

<Text will be added>

7.2	IAB output power

Detailed structure of the subclause is TBD.

7.3	Output power dynamics

Detailed structure of the subclause is TBD.

7.4	Transmit ON/OFF power

Transmit ON/OFF power requirements include two parts: transmitter off power and transmitter transient period. For IAB-DU, it’s straightforward to reuse BS’s both requirements because IAB-DU behaviour is the same as BS.

For IAB-MT, reusing BS or UE requirements for both OFF power and transient period are the candidates. Comparing BS OFF power and UE OFF power requirements, BS FR1 requirements are more stringent than UE but FR2 visa verse. It is unlikely there will be many IAB nodes in close proximity so a relaxed requirement compared to UEs should not cause a rise in overall interference in the system. Therefore, IAB-MT can reuse BS off power. Regarding transient period requirements, the general profiles for BS and UE are similar but FR2 BS transient period length is shorter than UE. UE has many specific profiles requirements to explicitly define the transient period location for different physical layers and different test cases. As IAB-MT implementation has less restriction than commercial UE, it is expected that IAB-MT can meet BS more stringent transient period length requirements. Thus IAB-MT general profile requirements can reuse BS requirements. For the UE specific profiles, they are not needed for IAB-MT because IAB-MT should know UE behaviour when different physical channels are transmitted.

Based the above analysis, both IAB-DU and IAB-MT reuse BS ON/OFF power requirements including the OFF power and transient period.

7.5	Transmitted signal quality

7.5.1 IAB-DU transmitted signal quality

As the IAB-DU behavior is very similar with BS, all of the transmitted signal quality requirements can be imported from BS. Therefore, the frequency error, modulation quality and time alignment error requirements in clause 6.5.1, 6.5.2, 6.5.3 for BS type 1-H in TS 38.104 [TBD] apply to IAB-DU type 1-H.

7.6	Unwanted emissions

Detailed structure of the subclause is TBD.

7.7	Transmitter intermodulation

For conducted transmitter intermodulation, it is agreed to reuse the base station framework for both IAB-DU and IAB-MT.

8	Conducted receiver characteristics

8.1	General

<Text will be added>

8.2	Reference sensitivity level

Detailed structure of the subclause is TBD.

8.3	Dynamic range

Detailed structure of the subclause is TBD.

8.4	In-band selectivity and blocking

Detailed structure of the subclause is TBD.

8.5	Out-of-band blocking

Detailed structure of the subclause is TBD.

8.6	Receiver spurious emissions

Detailed structure of the subclause is TBD.

8.7	Receiver intermodulation

Detailed structure of the subclause is TBD.

8.8	In-channel selectivity

For IAB-MT, similar as legacy NR UE, there is no RX in-channel selectivity requirement defined in TS 38.101, therefore it is agreed that no in-channel selectivity will be defined for IAB-MT.

9	Radiated transmitter characteristics

9.1	General

9.2	Radiated transmit power

Detailed structure of the subclause is TBD.

9.3	IAB OTA output power

Detailed structure of the subclause is TBD.

9.4	OTA output power dynamics

Detailed structure of the subclause is TBD.

9.5	OTA transmit ON/OFF power

According to the analysis in 7.4, both IAB-DU and IAB-MT reuse BS radiated ON/OFF power requirements including the OFF power and transient period.

9.6	OTA transmitted signal quality

9.6.1 IAB-DU OTA transmitted signal quality

As the IAB-DU behavior is very similar with BS, all of the transmitted signal quality requirements can be imported from BS. Therefore, the frequency error, modulation quality and time alignment error requirements in clause 9.6.1, 9.6.2 and 9.6.3 for BS type 1-O and  type 2-O in TS 38.104 [TBD] apply to IAB-DU type 1-O and type 2-O respectively.

9.7	OTA unwanted emissions

Detailed structure of the subclause is TBD.

9.8	OTA transmitter intermodulation

Detailed structure of the subclause is TBD.

9.8.1 IAB DU intermodulation

For FR1 the IAB node could be co-located if the IAB TDD pattern for transmission and receiving is the same for both IAB-DU and IAB-MT of both co-located nodes, i.e. just coordinating the UL/DL timeslots is not sufficient but the actual Tx and Rx time instants of both IAB-DU and IAB-MT need to be aligned. IAB DU intermodulation requirement shall be reused from BS spec.

Therefore IAB-DU type 1-O will reuse the framework of OTA TX intermodulation requirement defined in TS 38.104 section 9.8 for NR BS type 1-O.

There is no OTA TX intermodulation requirement for IAB-DU type 2-O as there is no TX IMD requirement for BS type 2-O.

    - 9.8.2 IAB MT intermodulation

The co-location scenario of IAB is listed below as:

-co-located with another IAB-Node in same band

This scenario shall be further considered as if the IAB-DU and IAB-MT of one IAB-Node don’t transmit simultaneously, there will exist the scenario that during UL timeslot one is transmitting in the backhaul link and the other is receiving in the access link, and correspondingly during DL timeslot one can be transmitting in the access link and another is receiving in the backhaul link. In this case, the excessive interference will prevent the two IAB nodes to be co-located. Hence co-location scenario can only be considered for the following case:

-co-located as IAB-DU and IAB-MT of one IAB-Node transmit simultaneously in the same band with same TDD pattern as 	the other co-located BS and/or IAB-Node.

- In case of two co-located IAB-Nodes, all IAB-MTs and IAB-Dus need to follow same TDD pattern.

In this case, the co-location scenario is similar to a base station co-located with another base station in same band, hence the intermodulation requirement shall be reused from BS spec.

-co-located with another IAB-DU/BS in another band

With the above co-location scenarios, it is proposed to use the framework of base stationTX intermodulation requirement for IAB MT with certain co-location scenarios. Considering the large coupling loss, there is no TX IMD requirement for IAB-MT type 2-O.

9.9	Beam correspondence for IAB-MT

Detailed structure of the subclause is TBD.

10	Radiated receiver characteristics

10.1	General

10.2	OTA sensitivity

Detailed structure of the subclause is TBD.

10.3	OTA reference sensitivity level

Detailed structure of the subclause is TBD.

10.4	OTA Dynamic range

Detailed structure of the subclause is TBD.

10.5	OTA in-band selectivity and blocking

Detailed structure of the subclause is TBD.

10.6	OTA out-of-band blocking

Detailed structure of the subclause is TBD.

10.7	OTA receiver spurious emissions

Detailed structure of the subclause is TBD.

10.8	OTA receiver intermodulation

For IAB-MT type 2-O, similar as legacy FR2 NR UE, there is no RX intermodulation requirement defined in TS 38.101-2, therefore it is agreed that no RX intermodulation is defined for FR2 IAB-MT.

10.9	OTA in-channel selectivity

For IAB-MT, similar as legacy NR UE, there is no RX in-channel selectivity requirement defined in TS 38.101, therefore it is agreed that no in-channel selectivity will be defined for IAB-MT.

11	IAB RRM requirements

Detailed structure of the subclause is TBD.

12	IAB EMC requirements

Detailed structure of the subclause is TBD.

12.1 IAB EMC Emission requirements

EMC emissions testing covers both conducted and radiated emissions.  . Test methods and levels for conducted emissions, harmonic and voltage fluctuations and flicker testing defined by IEC/CISPR are independent of the product (IAB) characteristics and features, including the operating frequency or the Radio Access Technology (RAT). It is agreed that the applicable requirements for EMC conducted emissions of IAB nodes are the ones defined for NR BS in TS 38.113.

EMC radiated emissions test involves measuring the electromagnetic field strength of the emissions that are unintentionally generated by a product (an IAB node in this case). Emissions are inherent to the switching voltages and currents within any digital circuit. In this case, the radiated emission requirements as well as the test methods that apply are defined by IEC and CISPR subcommittee in the Standard CISPR 32 [X]. Testing and limits of emissions intentionally generated by the BS are covered by ITU-R recommendations SM.329 [X] and SM.1539 [X].

  - 12.3 IAB EMC immunity requirements

12.3.1 Radiated Immunity requirements

  - 12.3.1.1 One enclosure case

The IAB node is a network node with similar outside radio frequency electromagnetic environment as base station. So in this case, it is reasonable to apply base station test levels to the IAB node.

  - 12.3.1.2 Different enclosure case

For different enclosure cases, the radiated immunity requirement apply for each enclosure. Still the test level as reusing the base station requirement applies as the electromagnetic environment is similar to BS.

  - 12.3.1.4 Summary

The base station requirement as 3V/m from 80MHz to 6000MHz is used as a starting point and the exclusion band for Radiated immunity test is FFS.

12.3.2 Other Immunity requirements

It is agreed that similar working environment as for a base station applies to an IAB node. So it is agreed to reuse the BS requirements for IAB node, for the following immunity requirements as listed below.

- ESD test

- Fast transient common mode

- RF common mode 0.15 – 80 MHz

- Voltage dips and interruption

- Surge

NOTE: 	Radiated immunity requirement is FFS.

13	IAB performance requirements

Detailed structure of the subclause is TBD.

14	IAB conformance testing aspect

Detailed structure of the subclause is TBD.

Annex <A> (normative):
<Normative annex for a Technical Report>

<Text will be added>

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | Tdoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-04 | RAN4#94bis-e | R4-2005458 |  |  |  | Initial TR skeleton endorsed in R4-1912889 Updated TR to incorporate below TPs R4-1916003 [IAB] TP to TR 38.xxx Co-location scenarios R4-1916166 [IAB] TP to TR 38.xxx System parameter R4-1914761 [IAB] TP to TR 38.xxx Architecture R4-2002487 TP for TR _RF Requirements reference points R4-2002488 TP for TR _Spec organization R4-2002493 [IAB] TP to TR 38.xxx Antenna assumptions R4-2002497 TP for TR _9 IAB OTA transmitter intermodulation R4-2005480 Update on section 4 and section 5 based on latest agreement R4-2003758 TP to TR 38.xyz: Addition of antenna model and parameters in sub-clause 6.2 R4-2005476 [IAB EMC] TP to TR IAB EMC immunity requirements | 0.1.0 |
| 2020-06 | RAN4#95e | R4-2006378 |  |  |  | Updated TR to incorporate below TPs approved during meeting: R4-2008768 TP to TR 38.809 – Correction of IAB-DU and IAB-MT permutation in subclause 4.8. R4-2008777 TP for TR 38.809: Transmit ON/OFF power R4-2006274 TP for TR 38.809: IAB-DU Transmitted signal quality R4-2008787 TP to TR : IAB RX IM requirement (section 8.7 and 10.8) R4-2008789 TP to TR: IAB ICS requirement (section 8.8 and 10.9) R4-2009062 [IAB RF] TP to TR 38.809 IAB TX IMD R4-2008730 TP to TR 38.809 on IAB EMC Emissions R4-2008731 [IAB EMC]TP to TR IAB EMC immunity requirements | 0.2.0 |