| 3GPP TR 38.851 V0.2.0 (2021-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; <User Equipment (UE) Further enhancements of NR RF requirements for frequency range 2 (FR2) (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Inter-band DL CA	8

4.1	Applicability of CBM/IBM for different CA configurations	8

4.2	Feasibility study for CA configurations within same frequency group	8

4.3	Feasibility study for CA configurations between different frequency groups based on CBM	8

4.4	UE requirements for CA configurations CA_n258A-n260A and CA_n257A-n259A based on IBM	8

4.5	UE requirements for CA configurations within the same frequency group based on CBM	8

5	Inter-band UL CA	8

5.1	Feasibility study for CA configurations within same frequency group based on IBM and CBM	8

5.2	Feasibility study for CA configurations between different frequency groups based on CBM	8

5.3	UE requirements for CA configuration CA_n257A-n259A based on IBM	8

6	UL gaps for self-calibration and monitoring	8

6.1	Phase 1	8

6.1.1	Study of the performance gain over the current baseline	8

6.1.2	Study of RF performance evaluation/testability related to UE self-calibration and monitoring	8

6.1.3	Study network impact of UE emissions during UL gap	8

6.2	Phase 2	9

6.2.1	Specification related aspects	9

6.2.1.1	UL gap configuration(s)	9

6.2.1.2	UE capability	9

6.2.1.3	UE interruptions	9

6.2.2	UE fall back behaviour i.e. if gaps are not available for UE requesting gaps	9

6.2.3	Release independence aspects	9

Annex <A> (informative): <Informative annex for a Technical Specification>	10

Annex <X> (informative): Change history	10

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is a technical report for Rel-17 WI Further enhancements of NR RF requirements for frequency range 2 (FR2).

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

# 4	Inter-band DL CA

## 4.1	Applicability of CBM/IBM for different CA configurations

Following agreements have been reached concerning applicability of CBM/IBM for different CA configurations.

  - IBM UE capability is applicable for all CA configurations R4-2016915

  - On “frequency group” R4-2016915

    - “frequency group” term shall not be defined in specification

  - If either CBM or IBM is concluded as infeasible for certain band combinations, it is reasonable to clearly state in the spec that only the requirements of feasible BM apply to these band combinations. If both CBM and IBM are concluded as feasible for certain band combinations, IBM/CBM is up to UE’s capability. R4-2016915

  - Typical inter-band CA deployment between bands in the same frequency group cannot be limited to co-located deployments R4-2007813

  - It is agreed that RAN4 will not label CBM or IBM as a default BM method for any band combination. Used beam management is based on UE capability. This issue is not discussed anymore in RAN4 R4-2103111

  - There are no deployment restrictions (Non-co-located/co-located) for network to configure inter-band DL CA for CBM UEs. R4-2103399

  - UE RF requirements for CBM shall be derived based on co-located deployment scenario only. R4-2103399

## 4.2	Feasibility study for CA configurations within same frequency group

## 4.3	Feasibility study for CA configurations between different frequency groups based on CBM

## 4.4	UE requirements for CA configurations CA_n258A-n260A and CA_n257A-n259A based on IBM

### 4.4.1	Operating bands for CA

NR inter-band carrier aggregation is designed to operate in the operating bands defined in Table 4.4.1-1, where all operating bands are within FR2.

Table 4.4.1-1: Inter-band CA operating bands in FR2

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n257-n259 | n257, n259 |
| CA_n258-n260 | n258, n260 |

### 4.4.2	Configurations for inter-band CA

Table 4.4.2-1: NR CA configurations for inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50 | 100 | 200 | 400 |  |
| CA_n257A-n259A | - | n257 | 50 | 100 | 200 | 400 | 0 |
|  |  | n259 | 50 | 100 | 200 | 400 |  |
| CA_n257A-n259G | - | n257 | 50 | 100 | 200 | 400 | 0 |
|  |  | n259 | CA_n259G |  |  |  |  |
| CA_n257A-n259H | - | n257 | 50 | 100 | 200 | 400 | 0 |
|  |  | n259 | CA_n259H |  |  |  |  |
| CA_n257A-n259I | - | n257 | 50 | 100 | 200 | 400 | 0 |
|  |  | n259 | CA_n259H |  |  |  |  |
| CA_n257A-n259J | - | n257 | 50 | 100 | 200 | 400 | 0 |
|  |  | n259 | CA_n259J |  |  |  |  |
| CA_n257A-n259K | - | n257 | 50 | 100 | 200 | 400 | 0 |
|  |  | n259 | CA_n259K |  |  |  |  |
| CA_n257A-n259L | - | n257 | 50 | 100 | 200 | 400 | 0 |
|  |  | n259 | CA_n259L |  |  |  |  |
| CA_n257A-n259M | - | n257 | 50 | 100 | 200 | 400 | 0 |
|  |  | n259 | CA_n259M |  |  |  |  |
| CA_n257G-n259A | - | n257 | CA_n257G |  |  |  | 0 |
|  |  | n259 | 50 | 100 | 200 | 400 |  |
| CA_n257G-n259G | - | n257 | CA_n257G |  |  |  | 0 |
|  |  | n259 | CA_n259G |  |  |  |  |
| CA_n257G-n259H | - | n257 | CA_n257G |  |  |  | 0 |
|  |  | n259 | CA_n259H |  |  |  |  |
| CA_n257G-n259I | - | n257 | CA_n257G |  |  |  | 0 |
|  |  | n259 | CA_n259I |  |  |  |  |
| CA_n257G-n259J | - | n257 | CA_n257G |  |  |  | 0 |
|  |  | n259 | CA_n259J |  |  |  |  |
| CA_n257G-n259K | - | n257 | CA_n257G |  |  |  | 0 |
|  |  | n259 | CA_n259K |  |  |  |  |
| CA_n257G-n259L | - | n257 | CA_n257G |  |  |  | 0 |
|  |  | n259 | CA_n259L |  |  |  |  |
| CA_n257G-n259M | - | n257 | CA_n257G |  |  |  | 0 |
|  |  | n259 | CA_n259M |  |  |  |  |
| CA_n257H-n259A | - | n257 | CA_n257H |  |  |  | 0 |
|  |  | n259 | 50 | 100 | 200 | 400 |  |
| CA_n257H-n259G | - | n257 | CA_n257H |  |  |  | 0 |
|  |  | n259 | CA_n259G |  |  |  |  |
| CA_n257H-n259H | - | n257 | CA_n257H |  |  |  | 0 |
|  |  | n259 | CA_n259H |  |  |  |  |
| CA_n257H-n259I | - | n257 | CA_n257H |  |  |  | 0 |
|  |  | n259 | CA_n259I |  |  |  |  |
| CA_n257H-n259J | - | n257 | CA_n257H |  |  |  | 0 |
|  |  | n259 | CA_n259J |  |  |  |  |
| CA_n257H-n259K | - | n257 | CA_n257H |  |  |  | 0 |
|  |  | n259 | CA_n259K |  |  |  |  |
| CA_n257H-n259L | - | n257 | CA_n257H |  |  |  | 0 |
|  |  | n259 | CA_n259L |  |  |  |  |
| CA_n257H-n259M | - | n257 | CA_n257H |  |  |  | 0 |
|  |  | n259 | CA_n259M |  |  |  |  |
| CA_n257I-n259A | - | n257 | CA_n257I |  |  |  | 0 |
|  |  | n259 | 50 | 100 | 200 | 400 |  |
| CA_n257I-n259G | - | n257 | CA_n257I |  |  |  | 0 |
|  |  | n259 | CA_n259G |  |  |  |  |
| CA_n257I-n259H | - | n257 | CA_n257I |  |  |  | 0 |
|  |  | n259 | CA_n259H |  |  |  |  |
| CA_n257I-n259I | - | n257 | CA_n257I |  |  |  | 0 |
|  |  | n259 | CA_n259I |  |  |  |  |
| CA_n257I-n259J | - | n257 | CA_n257I |  |  |  | 0 |
|  |  | n259 | CA_n259J |  |  |  |  |
| CA_n257I-n259K | - | n257 | CA_n257I |  |  |  | 0 |
|  |  | n259 | CA_n259K |  |  |  |  |
| CA_n257I-n259L | - | n257 | CA_n257I |  |  |  | 0 |
|  |  | n259 | CA_n259L |  |  |  |  |
| CA_n257I-n259M | - | n257 | CA_n257I |  |  |  | 0 |
|  |  | n259 | CA_n259M |  |  |  |  |
| CA_n258A-n260A | - | n258 | 50 | 100 | 200 | 400 | 0 |
|  |  | n260 | 50 | 100 | 200 | 400 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |  |  |  |  |

### 4.4.3	Reference sensitivity for inter-band CA

The inter-band requirement applies for all active component carriers. The throughput for each component carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.3.2 of TS 38.101-2 and A.3.3.2 of TS 38.101-2 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1) with peak reference sensitivity for each carrier specified in section 7.3.2 of TS 38.101-2, and relaxation ΔRIB,P,n applied  to peak reference sensitivity requirement. ΔRIB,P,n is specified in Table 4.4.3-1. The requirement on each component carrier shall be met when the power in the component carrier in the other band is set to its EIS spherical coverage requirement for inter-band CA specified in sub-clause 4.4.4.

For the combination of intra-band and inter-band carrier aggregation, the intra-band CA relaxation, ΔRIB, is also applied according to the clause 7.3A.2.1 and 7.3A.2.2 of TS 38.101-2.

Table 4.4.3-1: ΔRIB reference sensitivity relaxation for inter-band CA for power class 3

| NR CA bands | NR band | ΔRIB,P,n (dB) |
| --- | --- | --- |
| CA_n257-n259 | n257 | 4.0 |
|  | n259 | 4.0 |
| CA_n258-n260 | n258 | 3.5 |
|  | n260 | 3.5 |

### 4.4.4	EIS spherical coverage for inter-band CA

The inter-band CA requirement applies per operating band, for all active component carriers with UL assigned to one band and one DL component carrier per band. The requirement on each component carrier shall be met when the power in the component carrier in the other band is set to its EIS spherical coverage requirement for inter-band CA specified in this sub-clause.

The inter-band CA spherical coverage requirement for each power class will be satisfied if the intersection set of spherical coverage areas exceeds the common coverage requirement. Intersection set of spherical coverage areas is defined as a fraction of area of full sphere measured around the UE where both bands meet their defined individual EIS spherical coverage requirements for inter-band CA operation. The common coverage requirement is determined as <100-percentile rank> %, where ‘percentile rank’ is the percentile value in the specification of spherical coverage for that power class from clause 7.3.4 of TS 38.101-2. The requirement is verified with the test metric of EIS (Link=Beam peak search grids, Meas=Link angle).

The reference measurement channels and throughput criterion shall be as specified in clause 7.3A.2.3 of TS 38.101-2. The requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to that specified in clause 7.3.2 of TS 38.101-2.

Unless otherwise specified, the minimum requirements for reference sensitivity shall be verified with the network signalling value NS_200 (Table 6.2.3.1-1 of TS 38.101-2) configured.

The required spherical coverage EIS for each band in inter-band CA operation is given in clause 7.3.4 of TS 38.101-2 and modified by ΔRIB,S,n. The value of ∆RIB,S,n is defined in Table 4.4.4-1.

Table 4.4.4-1: ΔRIB,S,n EIS spherical coverage requirement relaxation for inter-band CA for power class 3

| NR CA bands | NR band | ΔRIB,S,n (dB) |
| --- | --- | --- |
| CA_n257-n259 | n257 | [4.0] |
|  | n259 | [4.0] |
| CA_n258-n260 | n258 | 3.5 |
|  | n260 | 3.5 |

### 4.4.5	Other receiver characteristics

For the FR2 inter-band DL CA configurations in Table 4.4.2-1, maximum input level, ACS, and in-band blocking requirement are band combination agnostic and the existing requirements in clause 7.4A.3, 7.5A.3, and 7.6A.2.3 of TS 38.101-2 shall be reused.

### 4.4.6	Release independence

Table 4.4.6-1 described the release independence of FR2 inter-band DL CA to be specified in clause 6.2 of TS 38.307.

Table 4.4.6-1: NR interband CA within FR2

| Feature | DL/UL | Maximum number of bands | CA BW Classes | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- |
| Inter-band CA configurations within NR FR2 | DL | 2 | B, C, D, E, F, G, H, I, J, K, L, M, O, P, Q | TDD | Rel-16 | Table B.4.2-1 |
|  | UL | 1 | B, C, D, E, F, G, H, I, J, K, L, M, O, P, Q | TDD | Rel-16 |  |

## 4.5	UE requirements for CA configurations within the same frequency group based on CBM

# 5	Inter-band UL CA

## 5.1	Feasibility study for CA configurations within same frequency group based on IBM and CBM

## 5.2	Feasibility study for CA configurations between different frequency groups based on CBM

## 5.3	UE requirements for CA configuration CA_n257A-n259A based on IBM

# 6	UL gaps for self-calibration and monitoring

## 6.1	Phase 1

### 6.1.1	Study of the performance gain over the current baseline

### 6.1.2	Study of RF performance evaluation/testability related to UE self-calibration and monitoring

### 6.1.3	Study network impact of UE emissions during UL gap

## 6.2	Phase 2

### 6.2.1	Specification related aspects

#### 6.2.1.1	UL gap configuration(s)

#### 6.2.1.2	UE capability

#### 6.2.1.3	UE interruptions

### 6.2.2	UE fall back behaviour i.e. if gaps are not available for UE requesting gaps

### 6.2.3	Release independence aspects

Annex <A> (informative):
<Informative annex for a Technical Specification>

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-04 | RAN4#98bis-e | R4-2105495 |  |  |  | FR2 inter-band CA for different frequency band groups with IBM (TP to TS 38.851) | 0.1.0 |
| 2021-08 | RAN4#100e | R4-2108914 |  |  |  | TP to TR 38.851 Applicability of CBM IBM for different CA configurations | 0.2.0 |