| 3GPP TR 38.869 V1.0.0 (2023-09) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on low-power wake up signal and receiver for NR (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

`

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	8

1	Scope	10

2	References	10

3	Definitions of terms, symbols and abbreviations	10

3.1	Terms	10

3.2	Symbols	11

3.3	Abbreviations	11

4	Introduction	12

5	Use cases & KPI	13

6	Evaluation Methodology	14

6.1	General performance metrics	14

6.2	Evaluation assumptions	15

6.2.1	General evaluation assumptions	15

6.2.2	Coverage evaluation assumptions	17

6.2.3	Link performance evaluation assumptions	19

6.3	power consumption model	23

6.3.1	Power model for Main Radio (MR)	23

6.3.2	Power model for LP-WUR (LR)	24

7	LP-WUR and LP-WUS Design	25

7.1	LP-WUS receiver architectures	25

7.1.1	General description of receiver types	25

A)	RF envelope detection	25

B)	Heterodyne architecture with IF envelope detection	26

C)	Homodyne/zero-IF architecture with baseband envelope detection	27

D)	FSK receiver	28

E)	OFDMA-based signals/channels detection	30

F)	Additional LP-WUR related designs	31

7.1.1a	Analysis of LP-WUR architectures	32

7.1.1a.1	OOK-1/2/4	32

7.1.1a.1.1	RF envelope detection	32

7.1.1a.1.2	Heterodyne architecture with IF envelope detection	33

7.1.1a.1.3	Homodyne/zero-IF architecture with baseband envelope detection	33

7.1.1a.2	FSK	33

7.1.1a.2.1	Parallel RF envelope detection	33

7.1.1a.2.2	Parallel heterodyne architecture	34

7.1.1a.2.3	Parallel homodyne architecture	34

7.1.1a.2.4	Heterodyne architecture with frequency to amplitude conversion	34

7.1.1a.2.5	Homodyne architecture with frequency to amplitude conversion	34

7.1.1a.3	OFDMA-based signal	34

7.1.1a.3.1	Time-domain correlation (without FFT)	35

7.1.1a.3.2	Frequency-domain correlation (with FFT)	35

7.1.1a.4	References	35

7.1.2	[RAN4 studies of the receiver]	35

7.2	LP-WUS design and L1 procedure	36

7.2.1	Signal design	36

7.2.1.1	Waveform	36

A)	MC-ASK waveform	37

B)	FSK waveform	39

C)	OFDM waveform	39

7.2.1.2	Bandwidth and location	40

7.2.1.3	LP-WUS content	40

7.2.2	L1 procedures	41

7.2.2.1	RRM measurements	41

7.2.2.2	Synchronization of LP-WUR	42

7.2.2.3	LP-WUS monitoring	42

7.2.2.4	Activation/Deactivation procedures of LP-WUS	42

7.2.2.5	Coverage	43

7.2.2.6	Additional L1 procedures for RRC CONNECTED mode	43

7.2.2.7	Additional L1 procedures for RRC IDLE/INACTIVE modes	44

7.2.2.8	Others	44

7.3	Higher-layer aspects	44

8	Evaluation Results	45

8.1	Power and latency evaluation	45

8.1.1	RRC IDLE/INACTIVE mode	45

8.1.1.1	Duty cycled LP-WUS	45

8.1.1.1.1	Collection of the results	46

8.1.1.1.1.1	LR (PWURON, R, FAR) = (<10unit, <=1%, <=1%)	46

8.1.1.1.1.2	LR (PWURON, R, FAR) = (<10unit, (1%,5%], <=1%)	47

8.1.1.1.1.3	LR (PWURON, R, FAR)  = (>=10unit, <=1%, <=1%)	48

8.1.1.1.1.3	Latency	48

8.1.1.1.2	Observations	50

8.1.1.2	Continuous monitoring results and results with duty cycle ratio = 50% LP-WUS	51

8.1.1.2.1	Collection of the results	52

8.1.1.2.1.1	LR (PWURON, FAR)  = (<= 1unit, <=1%)	52

8.1.1.2.1.2	LR (PWURON, FAR)  = ( > 1unit, <=1%)	53

8.1.1.2.1.3	Latency	53

8.1.1.2.2	Observations	54

8.1.1.3	Various LP-WUR relative power for duty-cycled monitoring	56

8.1.1.3.1	Collection of the results	58

8.1.1.3.1.1	LR (FAR, Reff, R) = (<=0.1%, <=0.1%, <=1%)	58

8.1.1.3.1.2	LR (FAR, Reff, R) = (<=1%, (0.1%,1%], <=1%)	59

8.1.1.3.1.3	LR (FAR, Reff, R) = (<=0.1%, <=0.1%, (1%,10%])	59

8.1.1.3.1.4	LR (FAR, Reff, R) = (<=1%, (0.1%,1%], (1%,10%] )	60

8.1.1.3.2	Observations	61

8.1.1.4	Compared with eDRX	63

8.1.1.4.1	Collection of the results	63

8.1.1.4.1.1	LP-WUS monitoring within PTW and legacy PO determination	63

8.1.1.4.1.2	LP-WUS monitoring not restricted in PTW and dynamic PO determination	64

8.1.1.4.2	Observations	65

8.1.1.5	RRM for LP-WUR duty-cycled monitoring	66

8.1.1.5.1	Collection of the results	67

8.1.1.5.1.1	LR (FAR, Reff, R) = (<=1%, <=1%, <=1%)	67

8.1.1.5.2	Observations	68

8.1.1.6	RRM for LP-WUR continuously monitoring	69

8.1.1.6.1	Collection of the results	69

8.1.1.6.1.1	LR (Reff, PWURON, FAR) = (<=1unit, <=1%, <=1%)	69

8.1.1.6.2	Observations	70

8.1.1.7	Impact by various MR sync/re-sync assumptions	71

8.1.1.7.1	Collection of the results	71

8.1.1.7.1.1	LR (Reff, R, PWURON, FAR) = (<=1%, <=1%,<= 4unit, <=1%)	71

8.1.1.7.1.2	LR (Reff, R, PWURON, FAR) = (<=1%, <=1%, >= 10unit, <=1%)	72

8.1.1.7.2	Observations	73

8.1.1.8	Impact by various MR ramp-up time/transition energy assumptions and sleep state	74

8.1.1.8.1	Collection of the results	74

8.1.1.8.1.1	LR (Reff, R, PWURON, FAR) = (<=1%, <=1%,<=4unit, <=1%)	74

8.1.1.8.1.2	LR (Reff, R, PWURON, FAR) = (<=1%, <=1%,>=10unit, <=1%)	75

8.1.1.8.2	Observations	77

8.1.2	RRC CONNECTED mode	78

8.1.2.1	XR traffic model	78

8.1.2.1.1	Collection of the results	78

8.1.2.1.1.1	DL only traffic	78

8.1.2.1.1.2	DL + UL traffics	81

8.1.2.1.2	Observations	84

8.1.2.2	FTP 3 and IM traffic model	85

8.1.2.2.1	Collection of the results	86

8.1.2.2.1.1	LP-WUS Scheme 1	86

8.1.2.2.1.2	LP-WUS Scheme 2	90

8.1.2.2.2	Observations	91

8.2	Coverage	94

8.2.1	Comparison between LP-WUS and NR reference channel	94

8.2.1.1	Summary of the performance gap between LP-WUS and NR reference channel	94

8.2.1.2	NR Coverage for comparison	99

8.2.2	Results for Urban	101

8.2.2.1	OOK, Urban, normal UE, PDCCH	102

8.2.2.2	OOK, Urban, Redcap UE, PDCCH	104

8.2.2.3	OOK, Urban, Normal UE, Msg.3	106

8.2.2.4	OOK, Urban, Redcap UE, Msg.3	108

8.2.2.5	FSK, Urban, Normal UE, PDCCH	110

8.2.2.6	FSK, Urban, Redcap UE, PDCCH	112

8.2.2.7	FSK, Urban, Normal UE, Msg.3	114

8.2.2.8	FSK, Urban, Redcap UE, Msg.3	115

8.2.2.9	OFDM, Urban, Normal UE, PDCCH	116

8.2.2.10	OFDM, Urban, Redcap UE, PDCCH	121

8.2.2.11	OFDM, Urban, Normal UE, Msg.3	122

8.2.2.12	OFDM, Urban, Redcap UE, Msg.3	123

8.2.3	Results for Rural	124

8.2.3.1	OOK, Rural, normal UE, PDCCH	125

8.2.3.2	OOK, Rural, Redcap UE, PDCCH	127

8.2.3.3	OOK, Rural, Normal UE, Msg.3	129

8.2.3.4	OOK, Rural, Redcap UE, Msg.3	130

8.2.3.5	FSK, Rural, normal UE, PDCCH	131

8.2.3.6	FSK, Rural, Redcap UE, PDCCH	131

8.2.3.7	FSK, Rural, Normal UE, Msg.3	131

8.2.3.8	FSK, Rural, Redcap UE, Msg.3	133

8.2.3.9	OFDM, Rural, normal UE, PDCCH	134

8.2.3.10	OFDM, Rural, Redcap UE, PDCCH	136

8.2.3.11	OFDM, Rural, Normal UE, Msg.3	139

8.2.3.12	OFDM, Rural, Redcap UE, Msg.3	141

8.3	LLS results observations	142

8.3.1	Timing error	142

8.3.1.1	Results for timing error (without sliding window)	142

8.3.1.1A	Observation for timing error (without sliding window):	146

8.3.1.2	Results for timing error (with sliding window)	146

8.3.1.2A	Observation for timing error (with sliding window):	148

8.3.2	Frequency error	149

8.3.2.1	Results for frequency error	149

8.3.2.1A	Observation for frequency error	155

8.3.3	Spectral efficiency	155

8.3.4	RSRP RRM measurement accuracy	160

8.3.4.1	Results for LP-SS RSRP RRM measurement, TDL-C	160

8.3.4.1A	Observation RSRP LP-SS, TDL-C:	163

8.3.4.2	Results for LP-SS RSRP RRM measurement, AWGN	163

8.3.4.2A	Observation RSRP LP-SS, AWGN:	166

8.3.4.3	Results for SSS RSRP RRM measurement, TDL-C	166

8.3.4.3A	Observation for SSS RSRP RRM measurement, TDL-C	168

8.3.5	RSRQ RRM measurement accuracy	169

8.3.5.1	Results LP-SS, AWGN	169

8.3.5.1A	Observation RSRQ LP-SS, AWGN:	170

8.3.5.2	Results LP-SS, TDL-C	170

8.3.5.2A	Observation RSRQ LP-SS, TDL-C:	173

8.4	Network power consumption	173

8.4.1	Collection of the results	173

8.4.2	Observations	174

8.5	System overhead	175

8.5.1	Collection of the results	175

8.5.1.1	RRC IDLE/INACTIVE	175

8.5.1.1.1	LP-WUS	175

8.5.1.1.2	LP-SS	194

8.5.1.2	RRC CONNECTED	197

8.5.2	Observations	198

8.6	References	200

9	 Conclusions	203

9.1	Power/Latency/Coverage/Overhead/Network Energy	203

9.2	LP-WUR architecture	205

9.3	LLS performance and LP-WUS design/L1 procedure	205

Annex 7.1.1a	Analysis of LP-WUR architectures	207

Annex 8.1	Power evaluation	207

Annex 8.2	Coverage	207

Annex 8.3	LLS results	207

Annex 8.4	Network power consumption	208

Annex8.4.1	[8A-11]	208

Annex8.4.2	[8A-13]	211

Annex8.4.3	[8A-3]	212

Annex8.4.4	[8A-5]	213

Annex 8.5	System Overhead	213

Annex 8.5.1	[8A-11]	213

Annex 8.5.2	[8A-13]	215

Annex 8.5.3	[8A-5]	217

Annex 8.5.4	[8A-16]	218

Annex 8.5.5	[8A-4]	219

Annex 8.5.6	[8A-3]	222

Annex 8.5.7	[8A-9]	224

Annex X:	Change history	226

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is intended to capture the output of study item for " Study on low-power wake up signal and receiver for NR" [2].

The study includes investigations to

- the impact of low-power wake-up signal and receiver , including power saving benefit, coverage, system overhead impact, network energy impact and other related aspects.

- the receiver architecture for low-power wake-up receiver and provide analysis for power consumption, noise figure and etc.

- L1 designs and procedures changes needed to support the wake-up signal and evaluations for the link performances.

- higher layer protocol changes needed to support the wake-up signals

- related RAN4 impacts

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP RP-222644: " Study on low-power wake up signal and receiver for NR".

[3]	3GPP TR38.840: “Study on User Equipment (UE) power saving in NR”

[4]	3GPP TR38.830: “Study on NR coverage enhancements”

[5]	3GPP TR38.838: “Study on XR (Extended Reality) evaluations for NR”

[6]	3GPP TR38.875: “Study on support of reduced capability NR devices”

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1]

ADC	Analog to Digital Converter

ASK	Amplitude Shift Keying

BB	Base Band

BLER	Block Error Rate

BPF	Band Pass Filter

BWP	Bandwidth part

CORESET	Control resource set

CRC	Cyclic redundancy check

CP-OFDMA	Cyclic Prefix-Orthogonal Frequency-Division Multiple Access

CSI	Channel state information

DCI	Downlink Control Information

DRX	Discontinuous Reception

DFT-S-OFDMA	Discrete Fourier Transform-Spread-Orthogonal Frequency-Division Multiple Access

eDRX	Extended DRX

EPRE	Energy Per Resource Element

FAR	False Alarm Rate

FCS	Frame Check Sequence

FSK	Frequency Shift Keying

FLL	Frequency Locked Loop

FFT	Fast Fourier Transform

FR1	Frequency range 1

FR2	Frequency range 2

IF	Intermediate Frequency

LP-WUS	Low Power-Wake Up Signal

LP-WUR	Low Power-Wake Up Receiver

LP-SS	Low Power- Synchronization Signal

LO	Local Oscillator

LNA 	Low Noise Amplifier

LPF 	Low Pass Filter

LR	LP-WUR

MDR	Miss Detection Rate

MC-ASK	Multiple Carrier-Amplitude Shift Keying

MC-FSK	Multiple Carrier-Frequency Shift Keying

MR	Main Radio

NF	Noise Figure

OOK	On-Off keying

OFDM	Orthogonal Frequency Division Multiplexing

PDCCH	Physical Downlink Control Channel

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

PDSCH	Physical Downlink Shared Channel

PRACH	Physical Random Access Channel

PEI	Paging Early Indication

PO	Paging Occasion

PTW	Paging Time Window

PLL	Phase Locked Loop

PAPR	Peak to Average Power Ratio

RRC	Radio Resource Control

RRM	Radio Resource Management

RLM	Radio Link Monitoring

RS	Reference Signal

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RTC	Real Time Clock

RF	Radio Frequency

SSB	Synchronization Signal Block

SINR	Signal to Interference plus Noise Ratio

SNR	Signal to Noise Ratio

SC	Subcarrier

UE	User Equipment

XR	Extended reality

# 4	Introduction

5G systems are designed and developed targeting for both mobile telephony and vertical use cases. Besides latency, reliability, and availability, UE energy efficiency is also critical to 5G. Currently, 5G devices may have to be recharged per week or day, depending on individual’s usage time. In general, 5G devices consume tens of milliwatts in RRC idle/inactive state and hundreds of milliwatts in RRC connected state. Designs to prolong battery life is a necessity for improving energy efficiency as well as for better user experience.

Energy efficiency is even more critical for UEs without a continuous energy source, e.g., UEs using small rechargeable and single coin cell batteries. Among vertical use cases, sensors and actuators are deployed extensively for monitoring, measuring, charging, etc. Generally, their batteries are not rechargeable and expected to last at least few years as described in TR 38.875. Wearables include smart watches, rings, eHealth related devices, and medical monitoring devices. With typical battery capacity, it is challenging to sustain up to 1-2 weeks as required.

The power consumption depends on the configured length of wake-up periods, e.g., paging cycle. To meet the battery life requirements above, eDRX cycle with large value is expected to be used, resulting in high latency, which is not suitable for such services with requirements of both long battery life and low latency. For example, in fire detection and extinguishment use case, fire shutters shall be closed and fire sprinklers shall be turned on by the actuators within 1 to 2 seconds from the time the fire is detected by sensors, long eDRX cycle cannot meet the delay requirements. eDRX is apparently not suitable for latency-critical use cases. Thus, the intention is to study ultra-low power mechanism that can support low latency in Rel-18, e.g. lower than eDRX latency.

Currently, UEs need to periodically wake up once per DRX cycle, which dominates the power consumption in periods with no signalling or data traffic. If UEs are able to wake up only when they are triggered, e.g., paging, power consumption could be dramatically reduced. This can be achieved by using a wake-up signal to trigger the main radio and a separate receiver which has the ability to monitor wake-up signal with ultra-low power consumption. Main radio works for data transmission and reception, which can be turned off or set to deep sleep unless it is turned on.

The power consumption for monitoring wake-up signal depends on the wake-up signal design and the hardware module of the wake-up receiver used for signal detecting and processing.

The study should primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables. Other use cases are not precluded, e.g.XR/smart glasses, smart phones.

As opposed to the work on UE power savings in previous releases, this study will not require existing signals to be used as WUS. All WUS solutions identified shall be able to operate in a cell supporting legacy UEs. Solutions should target substantial gains compared to the existing Rel-15/16/17 UE power saving mechanisms. Other aspects such as detection performance, coverage, UE complexity, should be covered by the evaluation.

The study item includes the following objectives:

- Identify evaluation methodology (including the use cases) & KPIs [RAN1]

  - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables

    - Other use cases are not precluded

- Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4]

- Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4]

- Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals [RAN2, RAN1]

- Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms, the coverage availability, as well as latency impact of low-power WUR/WUS. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1]

  - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

Use the following terminology for future discussion,

- Main radio (MR): the Tx/Rx module operating for NR signals/channels apart from signals/channel related to low-power wake-up

- LP-WUR (LR): The Rx module operating for receiving/processing signals/channel related to low-power wake-up.

# 5	Use cases & KPI

Editor’s note: The following SI Objective is included in this section.

- Identify evaluation methodology (including the use cases) & KPIs [RAN1]

  - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables

    - Other use cases are not precluded

The following characteristics for target use cases are considered in the study item:

- IoT cases including e.g., industrial wireless sensors, controllers, actuators and etc, including the following characteristics,

  - primary for small form devices

  - power-sensitive

  - static, nomadic or limited mobility

- Wearable cases including e.g., smart watches, rings, eHealth related devices, and medical monitoring devices etc.,

  - primary for small form devices,

  - power-sensitive

  - low/medium speed, FFS: high speed

- eMBB cases including e.g., XR/smart glasses, smart phones and etc.,

  - devices form is various and not restricted

  - power-sensitive

  - low/medium speed, FFS:  high speed

Note: other use cases/characteristics are not precluded if any.

The latency for LP-WUS/WUR are considered as follows

  - Latency for RRC CONNECTED mode can be up to tens of milliseconds.

  - Latency for RRC IDLE/INACTIVE mode can be in the order of seconds, for I-DRX use cases, and the latency can be larger for eDRX use cases.

Both RRC IDLE/INACTIVE and CONNECTED modes were studied as part of the LP-WUS/WUR SI.

# 6	Evaluation Methodology

Editor’s note: The section includes evaluation methodology for this SI

## 6.1	General performance metrics

For system impact analysis, the following performance metrics are considered to be provided,

Table 6.1-1 Performance metrics for system impact analysis

| Performance Metric | Note |
| --- | --- |
| System overhead | expressed as percentage of used part of all REs for LP-WUS (including guard band or time or others resource used for LP-WUR if any) among all resources Other assumptions related to the system overhead analysis can be reported, e.g., the LP-WUR raw data rate evaluated in the coverage evaluations. |
| Capacity impact | Evaluate the system capacity impact due to introducing of LP-WUS Note: it is for UEs which are in connected mode. Definition is the same as in XR TR. |
| NW power consumption / Energy Efficiency | Impact of LP-WUS/WUR operation on gNB energy consumption as performance metric in system impact analysis. |

For power and latency evaluation of the LP-WUS, the following performance metrics definitions provided for future study

Table 6.1-2 Performance metrics for power and latency evaluation

| Performance Metric | Note |
| --- | --- |
| Power consumption | Relative power consumption in units. The power consumption includes main radio and LP-WUR. For comparison, the relative power consumption and evaluation period for baseline schemes should also be provided, as well as the power saving gain (i.e., percentage of power consumption reduction of the proposed power saving scheme from the baseline scheme). |
| Latency | For IDLE/INACTIVE state, - the latency is the time interval between the data arrival time at the gNB and the time of the first PO UE can monitor the paging message - alternatively, if UE is not required to monitor a PO after wake-up, company to report detailed procedure and definition of the latency - sync/re-sync for main radio is included |
| UPT | The definition is the same as in TR38.840 Note: it is for connected mode purpose. |

- Companies to report baseline scheme, e.g., PO monitoring with i-DRX, e-DRX, with or without PEI

- Companies to report the power consumption / power saving gain considering the FAR impact, latency considering MDR impact

- Other performance metrics (e.g., mobility) can be reported by companies (if any)

## 6.2	Evaluation assumptions

### 6.2.1	General evaluation assumptions

RRC IDLE/INACTIVE evaluation assumptions

The following is assumed for RRC IDLE/INACTIVE evaluation,

Table 6.2-1 RRC IDLE/INACTIVE evaluation assumptions

| Parameters | Value |
| --- | --- |
| i-DRX cycle length | 1.28s and other values not precluded and reported by companies, consider both with PEI/ without PEI |
| e-DRX cycle length | 20.48s, 61.44s and other values not precluded, company to report which value(s) are used. Note: ‘ultra-deep sleep’ state can be assumed for eDRX whenever necessary for baseline UE |
| Number of POs in Paging Frame | 1 |
| Number of DRXs per PTW | 4 |
| Number of SSB before PO / PEI | 1, 2 or 3, (used for e.g., AGC adjustment, T/F tracking, serving cell and intra-F measurement) company to report which value(s) are used Note: the assumptions is for MR wakes from ‘Deep sleep’ |
| Sync/re-sync after ultra-deep sleep | companies to report the timeline of sync/re-sync and X value, X is the time for sync/re-sync |
| RRM Measurement | Company to report whether and how the RRM measurement is assumed, e.g., whether RRM performed by main radio or LP-WUR, whether RRM is relaxed or not. |
| LP-WUS monitoring | Option 1: continuously monitoring Option 2: discontinuously monitoring, with [T] ms as the period for complete an on-and-off cycle, and [D] ms as the active time for monitoring LP-WUS every cycle. |
| Traffic | Option 1 (baseline): - The traffic arrival is modeled as a Poisson Arrival Process where inter-arrival times are exponentially distributed, the mean arrival time is P = YREF / RE, REF, where - RE, REF= 1%, 0.1%, 0.01% or 0.001% and YREF = 1.28s - Per group paging probability RG = 1 – (1 – RE)N, where N is the number of UEs in the group - FFS: Value of N - For LP-WUS - Both per group and UE paging can be assumed. Note: - For i-DRX with cycle duration Y second, - Per UE paging probability RE = 1 – (1 – RE, REF )Y/YREF - For e-DRX with K i-DRX cycles duration, PTW duration of L i-DRX cycles, and an i-DRX cycle duration Y second - Per UE paging probability is - RE = 1 – (1 – RE, REF )(K-L+1)Y/YREF for the first i-DRX cycle within the PTW - RE = 1 – (1 – RE, REF )Y/YREF for each of the remaining L-1 i-DRX cycles within the PTW - L=4 Other options are not precluded can be reported by companies. |
| Others | Reported by companies |

The period of synchronization signal that LP-WUR used for at least power evaluation can be

- Existing SSB periodicity can be used from gNB transmission perspective for evaluations assuming SSB, companies to report how often used for LP-WUR

- For evaluations assuming LP-SS

  - {320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms}

  - Companies to report other important assumptions if any, e.g., durations of LP-SS to achieve enough T/F accuracy

- Other values are not precluded

Note: companies to report the purpose of the synchronization signal along with evaluations, e.g. can be for LR synchronization (i.e., time and/or frequency tracking) and/or measurement.

For reference setting for further study on LP-SS performance and resource overhead (including sync and/or measurement), companies to report the following used in their evaluations

- the number of slots or symbols per period

- the periodicity

- the functionality of the LP-SS

RRC CONNECTED mode evaluation assumptions

For R18 LP-WUS/WUR power evaluation in RRC connected mode, the following can be considered,

- XR traffic model with evaluation methodologies and assumptions captured in TR 38.838.

- eMBB traffic model with evaluation methodologies and assumptions captured in TR 38.840

- Heartbeat traffic models in 3GPP TR 38.875.

- Other models are not precluded.

Company to further provide the followings,

- Parameters (e.g., frame rate, data rate, jitter range, DRX configurations and etc if needed.)

- How to use LP-WUS, e.g., LP-WUS to trigger/adapt PDCCH monitoring

- Other details if any

For all RRC IDLE/INACTIVE/CONNECTED mode,

- Power pooling between OFDM symbols is not assumed for evaluation purposes. Average EPRE is defined per OFDM symbol.

### 6.2.2	Coverage evaluation assumptions

For evaluation of the coverage of LP-WUS, the methodology and assumptions in R17 CovEnh SI (described in TR38.830) are reused as baseline.

- MIL is used as the metric for LP-WUS coverage evaluation

- urban (2.6GHz/4GHz), rural(700MHz) scenario for FR1 are considered to be evaluated, others (e.g., FR2) are not precluded.

- Note: For IoT/wearables devices, refer to R17 Redcap SI TR38.875 if the assumptions differ from TR38.830.

- Companies report any other assumptions which differ from the TR38.875/ TR38.830, e.g., Tx and Rx loss

- Companies are encouraged to compare LP-WUS with at least PDCCH for paging, PUSCH, others are not precluded. FFS: Target coverage of LP-WUS

For LP-WUS coverage evaluation, the noise figure of LP-WUR is

  - Options : 9, 12, 15, 18, 21, 24 dB, Other values can be reported by companies

- FFS: how to determine the NF option.

- The values provided are for the purpose of studying coverage of LP-WUS, and they can be further revisited depending on the receiver architecture discussion.

For evaluation, 1 Rx chain for LP-WUS receiver is baseline.

For coverage evaluation, the following is used,

Table 6.2-1 coverage evaluation assumptions

| Number of RX chains at the UE’s MR | Case 1: 1 Rx for Redcap Case 2: 2 Rx Case 3: 4 Rx Company to report which case is being used. |
| --- | --- |
| Number of RX chains for LP-WUR | 1 Rx |
| Scenario and frequency | Urban: 4GHz (TDD), 2.6GHz (TDD) Rural: 700MHz (FDD) |
| Reference data rates for MR | Urban: PDSCH 10Mbps, PUSCH 1Mbps Rural: PDSCH 1Mbps, PUSCH 100kbps |
| Reference PDCCH configuration | SCS; 30kHz for TDD, 15kHz for FDD.. Aggregation level; 8, 16 Company to report which case is being used. Further decision on aggregation level for coverage is FFS.. Payload; 40 bits. CORESET size; 2 symbols, 48 PRBs. Tx Diversity; Reported by companies. BLER; 1% BLER,. |
| Pathloss model (select from LoS or NLoS) | Urban: NloS Rural: NloS |
| Bandwidth | 100MHz for 4GHz and 2.6GHz. 20MHz (optional for 10MHz) for 700MHz. (FDD) |
| Channel model for link-level simulation | TDL-C for NLOS |
| Delay spread | Urban: 300ns, optional: 1000ns and companies to provide descriptions for such scenarios Rural: 300ns |
| UE velocity | Urban: 3km/h Rural: 3km/h, FFS: 120km/h (optional 30km/h) for outdoor |
| Number of antenna elements for BS | - Urban: 192 antenna elements for 4GHz and 2.6GHz, (M,N,P,Mg,Ng) = (12,8,2,1,1) (optional) 128 antenna elements for 4GHz, (M,N,P,Mg,Ng) = (8,8,2,1,1) - Rural: 16 antenna elements for 700MHz (M,N,P,Mg,Ng) = (4,2,2,1,1) |
| Number of TxRUs for BS | gNB architectures to study: - 2 or 4 TXRUs for 700 MHz - 64TxRUs for 2.6 and 4 GHz. |

RAN1 further study the designs [target]/techniques of LP-WUS to have a comparable coverage as NR channel X. The NR channel X is

- Option #1: PDCCH for paging

- Option #2: PUSCH for message3

- FFS other options, e.g., between option1and option2 (better than PUSCH, worse than PDCCH)

- The final design will jointly consider the coverage with other KPIs

Use the same channel specific assumptions as defined in TR38.830 for reference PUSCH for message3, i.e.,

| Parameter | Value |
| --- | --- |
| Frequency hopping | w/ or w/o frequency hopping |
| Number of UE transmit chains | 1, 2 (optional) |
| Number of DMRS symbol | w/o frequency hopping: 3, w/ frequency hopping: 2 for each hop |
| Waveform | DFT-s-OFDM |
| SCS | 30kHz for TDD, 15kHz for FDD. |
| HARQ configuration | Whether HARQ is adopted is reported by companies. |
| PUSCH duration | 14 OS |
| Number of PRBs | 2 |
| TBS | 56 bits |
| Other parameters | Reported by companies. |

### 6.2.3	Link performance evaluation assumptions

For link-level simulation of LP-WUS, the following table is used as starting point,

- Note: The assumptions are not intended to limit the scope of the study or the design.

Table 6.2-2. Simulation assumptions for LP-WUS

| Attributes | Assumptions |
| --- | --- |
| Carrier Frequency | 2.6GHz/4GHz/700MHz |
| Waveform | OOK , FSK , OFDM Company to report which option for OOK /FSK /OFDM is used |
| Channel structure | - Option 1: Sync signal /sequence+ payload + CRC, - Option 2: Sequence only, - Option 3: Payload+CRC, - Other options are not precluded - Company to report the sequence length, payload size, CRC length (may or may not be presence). |
| SCS of OFDM generator for NR signal | 30kHz/15KHz |
| Configuration for LP-WUS signal | For OOK/FSK waveform, - Option 1a: M=1 and SCSs = 15kHz (same as NR signal) - Option 1b: M=1 and SCSs = 30kHz (same as NR signal) - Option 2a: M =2/4/8 for SCS = 15KHz (same as NR signal) - Option 2b: M =2/4/8 for SCS = 30 kHz (same as NR signal) - Option 3: M=1 and SCSs = 60kHz/120kHz/240kHz - Note: M is referred to the definition of “M” in the agreements for OOK-1/2/3/4 and FSK-1/2 For OFDM: FFS, e.g., ZC sequence Other options are up to companies to report |
| WUS duration | Number of OFDM symbols: e.g., 1,2,4, 8, 16,24 symbols |
| MDR/FAR assumption | - The miss-detection rate (MDR) of LP-WUS 1%, - The false-alarm rate (FAR) of LP-WUS - 0.1%, 1% - Other values are not precluded for studying, reported by companies - Note: if LP-WUS for wake-up indication consists of two parts or even multiple parts, the proposed MDR/FAR should take into account the reception performance of the two or more parts jointly - The above values are applied in both RRC CONNECTED and IDLE/INACTIVE mode. - FFS FAR requirement based on the study outcome of the impact of FAR on power consumption / power saving gain / system overhead The FAR definition does NOT include the impact of the falsely alarmed for wake-up due to the detection of a LP-WUS which is intended to wake-up/alarm the LP-WUR of another UE within the same UE group For evaluation purpose, FAR target is determined across a reference time duration T of one or multiple LP-WUS attempts/trials, - UE has N attempts within T, - Company to report (FAR target, T, N) - For example, - if UE makes a single decision based on multiple correlations for a sequence in the monitor occasion, these correlations are considered as UE implementation in ONE trial/attempt. - if UE performs decoding in a monitor occasion, a single decoding is considered as ONE trial/attempt. - If UE performs N non-overlap attempts within the reference time duration, the false alarm event for the attempts are assumed as independent. Companies to provide the assumed side conditions to attain the used FAR over T or per one attempt e.g. CRC/sequence length in LP-WUS design. |
| Code scheme | Companies to report, if any, the coding scheme (e.g., Manchester code or any other schemes) and the code rate (e.g., 1/2, 1/4, ….) |
| gNB Channel BW | 20MHz, FFS other values |
| LP-WUS BW | Option 1: - 5MHz including subcarriers for guard band - 4.32MHz (i.e.,12 RBs) for LP-WUS transmission for 30kHz SCS Option 2: - {2.16, 4.32} MHz including subcarriers for guard band - 1.44MHz, 2.88MHz (i.e.{4, 8} RBs) for LP-WUS transmission for 30kHz SCS Other options are up to companies to report GB is symmetrically placed on each side of LP-WUS |
| Filter | X-th Order filter (e.g. Butterworth, Chebyshev, …) with Y MHz bandwidth, - X = {3, 5} - Companies to report Y Companies to report any other assumptions if needed |
| Adjacent subcarrier interference | - PDSCH mapped on resources other than that for WUS and guard band; EPRE of LP-WUS / EPRE of PDSCH =ρ, where ρ=0 dB as baseline, ρ= {3, 6} dB as optional |
| Sampling Rate | - Companies to report. |
| ADC bit width | 1-bit, 4-bit, 8-bit, ideal and other options are not precluded |
| Channel Model | See link coverage assumption table (Refer to Table 6.2-1) |
| Impairment modelling | - Frequency and time error model is referred to Table 6.2-3 and 6.2 -4 - Phase noise up to company report, e.g. the modelling used for 802.11ba - Other cell interference is up to company to report |

- For evaluation of LP-WUR frequency and time errors, the following is used,

Table 6.2-3. Freuqnecy error/drifting

| Parameter | Value |
| --- | --- |
| Oscillator max frequency error [ppm], Oscillator frequency drift [ppm/s] | option 1: (200, 0.1) option 2: (50, 0.1) option 3: (10, 0.05) option 4: (5, 0.05) Other values are not precluded for studying, reported by companies |
| RTC max frequency error [ppm], | 20 RTC drift report by company |

- Company to report how to  use the clocks for LR on/off states

  - The above clock assumptions for LR assume the MR is in ‘ultra-deep sleep’ power state.

  - Note: Assumptions important for achieving performance by option 1/2/3/4 clock for LR should be declared, including active on/off power, transition energy/ ramp-up time TLR, ramp-up for LR and etc.

  - If MR is in other states than ‘ultra-deep sleep state’, the clock running for MR can be used for LR.

    - assumptions important for achieving performance by using MR clock for LR should be declared

  - Other clock accuracy options are not precluded. Companies to report options based on a feasibility analysis of clock power consumption and UE power consumption to use the clock accuracy option

- Company to report the frequency error assumption for the detection of LP-WUS/synchronization signal,

  - The following are examples for consideration, other approaches are not precluded,

    - Model 1:

      - The relationship between a drifted frequency error(ΔF), frequency drift ( F’) over a time (T1) is ΔF = ±F’ * T1

      - When frequency displacement [Fd] reaches max frequency error, it is assumed to be equaled to max frequency error

      - T1 is the time from the previous frequency synchronization. T1 may take different values depending on the chosen frequency synchronization approach.

      - For Model 1 of frequency error, Frequency displacement (Fd), defined as the difference between ideal frequency and frequency due to 1) clock drifting (ΔF); and 2) residual frequency error from previous synchronization/calibration (Fr), is given as Fd (ppm)=ΔF (ppm) +Fr(ppm),

        - Companies to report Fr and important assumptions for achieving Fr, e.g., if MR can assist to calibrate LP-WUR to correct the frequency error or if LP-WUR can only correct the frequency error based on LP-WUS synchronization signal

- Company to report the timing drifting error assumption for the detection of LP-WUS/synchronization signal,

  - The following are examples for consideration, other approaches are not precluded,

    - Model 1 [R1-2301438] [R1-2301558][R1-1714993]:

      - The relationship between the maximum frequency error(Fe) and corresponding timing drift( ΔT) over a time(T) is ΔT = ±Fe * T (linear region)

      - The relationship between a frequency drift( F’), and corresponding timing drift(ΔT) over a time(T) is ΔT = Fr*T ±0.5 * F’ *T2 (transient region)

      - The transition between transient and linear region (from synchronization or calibration point/time) occurs at time [Ts= (Fe-Fr)/( F’)]

      - T is the time from the previous time synchronization. T may take different values depending on the chosen synchronization approach

      - FFS: Time error (Te) before detection of a current sync signal is defined as the difference between ideal time of the current sync signal and the time error due to 1) clock time drift (ΔT); and 2) residual time error from previous synchronization/calibration (Tr); Te= ΔT+ Tr

The following for usage of the clock is assumed for LP-WUR OFF/ON

Table 6.2-4. Assumptions for the usage of the LP-WUR clock

| Assumption on LP-WUR OFF power | Assumptions on the clock usage |
| --- | --- |
| 0.001 | When LP-WUR is OFF - Time offset cumulated in the off period cannot be calculated based on the parameters of the oscillator option 1/2/3/4. RTC should be used(Only RTC is running during sleep.) When LP-WUR is ON, frequency offset and time offset calculation can follow the parameters of the oscillator option 1/2/3/4 (cumulating based on the frequency drift and not exceeding maximum frequency error) - The initial frequency offset when LP-WUR switches on can be set to the [FFS: maximum frequency error or a random value within the maximum frequency error] following the parameters of the oscillator option 1/2/3/4. - When LP-WUR is synced with LP-SS/SSB or MR is used to assist to calibrate LP-WUR to correct the time/frequency error, residual frequency error Fr is assumed at the time when the synchronization/calibration is done. |
| >0.001 | For both LP-WUR OFF and ON - Time offset cumulated in the off period can be calculated based on the parameter of the oscillator option 1/2 or option 3/4. RTC can be used too. - Frequency offset calculation can follow the parameter of the oscillator option 1/2 or option 3/4 (cumulating based on the second value in the value pair and not exceeding maximum frequency error). When at the time point after LP-WUR is synced with LP-SS/SSB or if MR can assist to calibrate LP-WUR to correct the frequency error - Frequency offset is the Fr, which is residual frequency error from previous synchronization/calibration |

Note1: Any additional LO/FLL/PLL could start running during LP-WUR On duration. The power consumption of any of those LO/FLL/PLL is captured in LP-WUR On power

Note3: The clock error (of both RTC and LO) could be improved to be less than max ppm error of option 1,2,3,4 with clock calibation based on sync signal such as LP-SS or preamble.

## 6.3	power consumption model

### 6.3.1	Power model for Main Radio (MR)

Take the following power model for main radio for evaluation in LP-WUS/WUR SI,

- For IoT and wearable cases, reuse TR38.875 power model as baseline.

- For eMBB and other cases, reuse TR38.840 power model as baseline.

- Introduce ‘Ultra-deep sleep’ power state for main radio of UEs with LP-WUS receiver

The following power models are used for ‘Ultra-deep sleep’ power state for main radio for evaluation

Table 6.3-1. Power model for Main Radio

| Power State | Relative Power (unit) | Ramp-up and down transition energy (Note1): (unit multiplied by ms) | Ramp-up time | Time for sync/re-sync |
| --- | --- | --- | --- | --- |
| Ultra-deep sleep | 0.015 | For evaluation, at least for FR1 MR ultra-deep sleep state, (Ramp-up and down transition energy, ramp-up time) is as follows, - Alt 1: (15000, 400ms) as baseline - Alt 2: (40000, 800ms) Company to report which alternative they use for which use cases. |  | For MR, at least for FR1 evaluation, - Number of SSBs for sync/re-sync for MR is up to 10 - Companies to report timeline and energy consumption |

Note1:

  - Ramp-up time may consist of the procedure for main radio hardware tune on e.g., boot, memory load and etc.,

  - Time for sync/re-sync consists of the procedure for main radio to re-synchronization with the serving gNB etc.,

    - Companies can report the assumption of X in the initial evaluation.

  - Ramp up and down energy includes power for ramp-up and ramp-down. Energy consumption for sync/re-sync is separately calculated.

- The total time for main radio transition from ultra-deep sleep to active/micro sleep state is the sum of ramp-up time and time for sync/re-sync.

Note 2: the power state transitions in this table refer to transitions between ultra deep sleep state and active / micro sleep state.

### 6.3.2	Power model for LP-WUR (LR)

The following power model for LP-WUR is used for evaluation for FR1,

Table 6.3-2. Power model for LP-WUR

| Power State | Relative Power (unit) | Additional transition energy: (unit multiplied by ms) | Ramp-up time TLR, ramp-up (ms) |
| --- | --- | --- | --- |
| Off[1] | 0.001/ 0.02/ 0.1, For 0.1, [oscillator option 3/4] are not used for envelope detection based receiver | TLR, ramp-up *(PON-POFF)/2 Note: this assumes the power consumption during the transition time is sum of additional transition energy and LP-WUR OFF energy, e.g., similar definition as the additional transition energy in TR38.840 | company to report TLR, ramp-up |
| On[2] | 0.01/0.05/0.1/0.2/0.5/1/2/4/10/20/30 |  |  |

- FFS: whether further categorization/sub-categorization is needed and how.

- FFS: Mapping from values to a LP-WUR architecture or LP-WUR mode of operation

- For evaluation, 10/20/30 for LP-WUR ON power state are not used for envelope detection based receiver for LP-WUS monitoring.

- For evaluation, 10/20/30 for LP-WUR ON power state are used for OFDM receiver when noise figure is less than [MR noise figure + 2.5dB], [0.2/0.5/1/2/4] for LP-WUS can be assumed for other NF values larger than [MR noise figure + 2.5dB]

- FFS: LP-WUR power consumption values for FR2.

- Note1: A unit of power is defined to be the same for main receiver and LP-WUS receiver.

- Note2: The values provided are for the purpose of studying power saving gain, and the values can be further revisited and categorized depending on the receiver architecture discussion.

- Note3: For LP-WUR ‘on’ state, more than one value within the above range may be used for evaluation (e.g. for a single LP-WUR architecture)

- Note4:

  - For WUR Off value 0.001, oscillator options 1, 2, 3, 4 are not assumed and only RTC is maintained;

  - [For WUR Off value 0.02, oscillator options 1, 2 and RTC can be maintained; ]

- Note5: Up to companies to report whether same or different values are assumed for WUS monitoring and time/frequency synchronization.

[1] Relative power unit for LP-WUR ‘off’ state, i.e., the LP-WUR does not perform monitoring

[2] Relative power unit for LP-WUR ‘on’ state, i.e., the LP-WUR performs monitoring

# 7	LP-WUR and LP-WUS Design

## 7.1	LP-WUS receiver architectures

Editor’s note: The following SI Objective is included in this section.

- Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4]

This section includes the studies for LP-WUR architectures. It does not intend to mandate the implementation of any specific type(s) of LP WUR architecture at the UE. Note this does not prevent RAN4 from defining requirements for LP WUR in the normative phase.

For the study on LP WUR architecture, power consumption relative to the deep sleep state of the MR is provided.

- Deep sleep state of non-RedCap UE should be assumed

For the LP WUR architectures analysis, in addition to LP-WUS detection, consider the following functions when necessary:

- Synchronization signal processing and time/frequency synchronization for LP-WUR

- RRM measurement at least for the serving cell

### 7.1.1	General description of receiver types

Editor’s note: Any general description of the receiver types from RAN 1/RAN4 related conclusions

Study at least the following three types of receiver architectures for LP-WUR:

- Architecture with RF envelope detection

- Heterodyne architecture with IF envelope detection

- Homodyne/zero-IF architecture with baseband envelope detection

- Note: The details of each type of receiver architecture are discussed separately.

- Note: Above receiver architectures are considered suitable for OOK modulation. Some of the architectures

can be applicable for other modulations such as FSK.

#### RF envelope detection

The architecture with RF envelope detection is based on at least the following diagram for LP-WUR.

- The RF signal is converted into baseband signal directly via an RF envelope detector.

- There is no Local Oscillator (LO) and no Phase-Locked Loop (PLL).

- 1 bit or multi-bit ADC is applied.

- Some component(s), e.g., RF LNA and/or BB AMP, can be optionally applied.

- High-Q matching network and/or RF BPF [and/or BB LPF] can be used to suppress adjacent channel interference or interference from legacy NR signals and/or other LP WUS on adjacent subcarriers.

- FFS the support of band and/or carrier tuning

Figure 7.1.1-1 RF envelope detection based LP-WUR diagram

For the architecture with RF envelope detection,

- It can achieve relatively low power consumption due to the removal of LO/PLL.

- Interference suppression for adjacent channel interference requires very high-Q matching network and/or RF BPF, which is challenging due to the high Q values and may require off-chip components.

- Interference suppression for interference from legacy NR signals and/or other LP WUS on adjacent subcarriers, if performed in RF, requires very high-Q matching network and/or RF BPF, which is challenging due to the high Q values and may require off-chip components.

- The support of multiple bands and/or carriers may require multiple high-Q matching networks and/or RF BPFs or multiple off-chip components.

- RF LNA can be applied to improve sensitivity, with the cost of additional power consumption.

- The noise figure can be relatively high.

#### Heterodyne architecture with IF envelope detection

The heterodyne architecture with IF envelope detection is based on at least the following diagram for LP-WUR.

- The RF signal is down converted into IF signal via an RF mixer with a LO. The IF signal is converted into baseband signal via an IF envelope detection.

- There may be one or multiple IF stages depending on design.

- The choice of the LO is one of the major factors that determine the power consumption.

- Lower power consumption can be achieved by relaxing the accuracy and stability requirements of the LO. However, such increased frequency offset and phase noise should be taken into account in the design and evaluation.

- FLL (frequency locked loop) may replace PLL for non-coherent detection.

- 1-bit or multi-bit ADC is applied.

- High-Q matching network and/or RF BPF and/or IF BPF [and/or BB LPF] can be used to suppress adjacent channel interference or interference from legacy NR signals and/or other LP WUS on adjacent subcarriers.

- Some component(s), e.g., RF LNA and/or IF AMP and/or BB AMP, can be optionally applied.

- Image rejection filter or an image rejection mixer is required.

- FFS the support of band and/or carrier tuning

- FFS the choice of IF frequency range

Figure 7.1.1-2 Heterodyne architecture with IF envelope detection based LP-WUR diagram

For heterodyne architecture with IF envelope detection,

- For the support of band and/or carrier tuning, the band and/or carrier tuning can be achieved via tuning the LO frequency.

- The matching network and RF BPF for LP WUR may or may not reuse those of the main radio.

- It is more effective and less complex to use IF BPF instead of high-Q matching network and/or RF BPF to suppress adjacent channel interference or interference from legacy NR signals and/or other LP WUS on adjacent subcarriers.

- Using FLL instead of PLL consumes less power, but it may result in larger frequency error.

- The IF frequency can be properly selected to avoid LO leakage (DC offset) and flicker (1/f) noise.

- Image rejection can be done via either image rejection filter or image rejection mixer.

  - Image rejection filter can be done in either RF or IF, which may require high-Q filter.

  - Image rejection mixer requires two-branch (I/Q) mixing with good matching in gain and phase, which consumes additional power.

- RF LNA and/or IF AMP can be applied to improve sensitivity, with the cost of additional power consumption.

#### Homodyne/zero-IF architecture with baseband envelope detection

The homodyne/zero-IF architecture with baseband envelope detection is based on at least the following diagram for LP-WUR.

- The RF signal is directly down converted into baseband signal via an RF mixer with a LO.

- Baseband envelope detection can be done either in analog domain or in digital domain depending on design, which is not explicitly shown in the diagram.

- The choice of the LO is one of the major factors that determine the power consumption.

- Lower power consumption can be achieved by relaxing the accuracy and stability requirements of the LO. However, such increased frequency offset and phase noise should be taken into account in the design and evaluation.

- FLL (frequency locked loop) may replace PLL for non-coherent detection.

- 1-bit or multi-bit ADC is applied.

- High-Q matching network and/or RF BPF and/or BB BPF [and/or BB LPF] can be used to suppress adjacent channel interference or interference from legacy NR signals and/or other LP WUS on adjacent subcarriers.

- No image rejection filter is required.

- Some component(s), e.g., RF LNA and/or BB AMP, can be optionally applied.

- FFS the support of band and/or carrier tuning

Figure 7.1.1-3 Homodyne/zero-IF architecture with baseband envelope detection based LP-WUR diagram

For homodyne/zero-IF architecture with baseband envelope detection,

- For the support of band and/or carrier tuning, the band and/or carrier tuning can be achieved via tuning the LO frequency.

- The matching network and RF BPF for LP WUR may or may not reuse those of the main radio.

- It is more effective and less complex to use BB BPF/LPF instead of high-Q matching network and/or RF BPF to suppress adjacent channel interference or interference from legacy NR signals and/or other LP WUS on adjacent subcarriers.

- Using FLL instead of PLL consumes less power, but it may result in larger frequency error.

- It can suffer from LO leakage (DC offset) and flicker (1/f) noise. The impact may be alleviated by using BB BPF in some cases.

- RF LNA can be applied to improve sensitivity, with the cost of additional power consumption.

- The baseband envelope detection can be done in either analog domain (before ADC) or digital domain (after ADC).

#### FSK receiver

Two examples for FSK receiver architectures are shown below:

- Example 1: parallel OOK receivers and a comparator circuit, e.g.,

  - Each path can be implemented using either of [the architecture with RF envelope detection,] heterodyne architecture with IF envelope detection, or homodyne/zero-IF architecture with baseband envelope detection.

Figure 7.1.1-4 FSK receiver with parallel OOK receivers and a comparator circuit based LP-WUR diagram

Study the parallel receiver architectures (as examples that can be captured in the TR) for FSK based on the following diagrams:

- Parallel homodyne architecture

  - The observations made for homodyne/zero-IF architecture with baseband envelope detection in section 7.1.1-(C) are also applicable here.

- Parallel heterodyne architecture

  - The observations made for heterodyne architecture with IF envelope detection in section 7.1.1-(B) are also applicable here.

- Note: Other architectures are not precluded.

- The OOK receiver architectures agreed for study in RAN1#110bis-e are also examples that can be captured in the TR

For FSK receiver based on parallel OOK receivers with heterodyne or zero-IF architecture,

- If no interference between the segments’ detectors is allowed, the frequency gap between two adjacent frequency segments should not be smaller than two times the maximum frequency offset, and at least two times of the max frequency offsets within the frequency gap should not be used by other DL signals/channels or other WUS signals.

- If some interference between the segments’ detectors is allowed, it can be possible to have frequency gap between two adjacent frequency segments smaller than two times the maximum frequency offset, where the gap is not used by other DL signals/channels or other WUS signals.

- Example 2: using an FM-to-AM detector [or an FM detector]

  - Alt 1: Use an analog FM-to-AM detector with a similar architecture as for OOK (e.g. heterodyne or zero-IF architecture), except that the envelope detector is replaced by a FM-to-AM detector.

    - Analog FM-to-AM detector can be implemented at least in BB or low-IF.

Figure 7.1.1-5 FSK LP-WUS receiver with FM-AM detector- Alt 1

  - Alt 2: Use a FM-to-AM detector [or an FM detector] implemented in digital domain after ADC, with a heterodyne or zero-IF architecture.

    - Digital FM-to-AM detector implementation can be considered as part of digital baseband processing.

    - Here is an example of using zero-IF architecture:

Figure 7.1.1-6 FSK LP-WUS receiver with FM-AM detector - Alt 2

  - The FM-AM detector can be implemented using a frequency discriminator, which converts frequency variations into amplitude changes. It can be implemented in either analog domain (as in Alt 1) or digital domain (as in Alt 2).

    - One example, as shown in the figure below, is a conventional quadrature FM discriminator. It multiplies received frequency modulated signal with a phase shifted version, followed by a low pass filter. The amplitude of the output signal is proportional to the frequency of the input signal.

Figure 7.1.1-7 FSK LP-WUS receiver with FM-AM detector by using a frequency discriminator

- Note: Other architectures are not precluded.

Study the receiver architectures (as examples that can be captured in the TR) for FSK with frequency to amplitude conversion based on the following diagrams:

- Homodyne architecture with frequency to amplitude conversion

  - I/Q branches are required for frequency to amplitude conversion in digital BB.

- Heterodyne architecture with frequency to amplitude conversion

  - Companies provide the exact type of frequency to amplitude conversion being studied.

- Note: Other architectures are not precluded.

Observation for FSK with frequency to amplitude conversion:

- The FSK architectures with frequency to amplitude conversion is applicable to single-SC FSK, but it may be challenging to make the frequency to amplitude conversion work well with multi-subcarrier FSK.

  - Note: single-SC FSK refers to the waveform where each frequency segment has a single subcarrier, and multi-subcarrier FSK refers to the waveform where each frequency segment has multiple subcarriers, as described in the agreements for FSK-1 and FSK-2.

For the FSK architectures with frequency to amplitude conversion, the bandwidth between the frequency segments used for FSK transmissions may not be used for other LP-WUSs or legacy NR transmission in order to allow frequency to amplitude conversion to work properly.

#### OFDMA-based signals/channels detection

For OFDMA-based signals/channels, study the receiver architectures based on the following diagrams:

- I/Q branches are required for digital BB processing.

- Digital BB processing may or may not include FFT (companies to provide details on how).

- For sequence-based OFDM signals/channels, digital BB processing includes sequence correlation in either time domain (without FFT) or frequency domain (after FFT).

- Proponent companies should at least provide details on power consumption reduction compared to the MR regarding the RF and digital BB processing.

  - Companies are encouraged to provide the break-down for the components.

  - The potential power reduction compared to the main radio may come from e.g.:

    - Lower performance LNA/amplifier

    - Oscillator/PLL with relaxed performance requirements

    - ADC with lower sampling rate and smaller bit-width

    - Reduced BB processing complexity compared to the MR

  - Companies are encouraged to provide the performance analysis corresponding to the considered power consumption considering the impact of e.g. phase noise, I/Q mismatch.

  - Companies to report whether the LP WUR is assumed to share components with MR. In case of component sharing, the potential impact on the MR ultra-deep sleep state should be considered.

  - Companies to report the possible number of information bits

- In addition, companies should consider the power consumption in the OFF state and the transition energy.

For the receiver architecture for OFDMA-based signals/channels,

- The receiver architectures for OFDMA-based signals/channels can be used for OOK/ASK and FSK modulated LP-WUS

- For sequence-based OFDM signals/channels, one example diagram with time domain correlator (without FFT) for LP-WUS detection is shown below:

#### Additional LP-WUR related designs

For the baseband processing of the LP WUR architectures,

- The baseband processing may use Goertzel filters as an alternative for FFT to compute the signals for one or more tones. Tone energy is computed and a detection algorithm is used to detect the presence of LP-WUS. One example diagram is shown below:

  - This can be used with the receiver architecture for OFDMA-based signals/channels for OOK-3.

  - This can be used with heterodyne receiver architecture with IF envelope detection or the homodyne receiver architecture with baseband envelope detection for [OOK-1]/FSK-2.

### 7.1.1a	Analysis of LP-WUR architectures

For the analysis of different LP-WUR receiver types, at least the following aspects have been considered, and the corresponding power consumption and noise figure have been estimated.

- Assumed modulation/waveform/coding

- Presence of a RF LNA / IF AMP / BB AMP, and the corresponding gain, if any

- Local oscillator

  - Type of oscillator and the corresponding frequency accuracy/drifting

- Handling of time/frequency impairments (e.g. synchronization signal processing and time/frequency synchronization for LP-WUR)

- Presence of PLL or FLL

- ADC: sampling rate, bit-width

- Assumed signal bandwidth and guard band, and frequency location within a carrier (including whether it is fixed or can be flexible)

- RF/IF/BB filter characteristics (e.g. type of filter, order, cut-off frequency/frequencies), if any

- Baseband processing (e.g., sequence correlation detection / decoding, other signal processing, if any)

- Assumed frequency band(s) and the support of band and/or carrier tuning

- Duty cycle handling of WUS and other signals (if any)

- Interference rejection capability (including both adjacent-channel interference and interference from adjacent subcarriers occupied by legacy NR signals or other LP WUS)

- Handling of inter-cell interference

- Whether there is any mobility support function, e.g. measurement capability

The results are summarized for each receiver architecture considering different waveform candidates (more details can be found in Annex 7.1.1a and the references are in 7.1.1a.4). The analysis is very high level, and the actual power consumption and noise figure are very implementation specific. Note that the power consumption is provided in relative unit, relative to the deep sleep state of the MR for a non-RedCap UE following the non-RedCap UE power model defined in section 8.1 of TR 38.840.

For the analysis,

- FR1 is considered.

- Most sources assumed RF LNA is present for improved sensitivity.

- For LP-WUS bandwidth, most sources consider at least 5MHz. Some sources think the analysis is also applicable to bandwidth up to 20MHz.

#### 7.1.1a.1	OOK-1/2/4

##### 7.1.1a.1.1	RF envelope detection

For OOK-1/2/4 with RF envelope detection, some sources ([7A-1] [7A-2] [7A-3] [7A-4] [7A-5] [7A-6]) provided analysis, with relative power consumption and noise figure summarized in Table 7.1.1a-1.

- The relative power consumption for ON state is in the range of 0.01~0.2.

- The noise figure is in the range of 12~22 dB.

Table 7.1.1a-1 Relative power consumption and noise figure for OOK-1/2/4 with RF envelope detection

| Source reference | [7A-1] | [7A-2] | [7A-3] | [7A-4] | [7A-5] | [7A-6] |
| --- | --- | --- | --- | --- | --- | --- |
| Power consumption (ON state) | 0.05 for single-branch, 0.01 for each additional branch | 0.01 | 0.01~0.1 | 0.01 | 0.01~0.1 | 0.05~0.2 |
| Noise figure (dB) | 20 | 17~22 | [12-18] | 20 | 15 | 20 |

##### 7.1.1a.1.2	Heterodyne architecture with IF envelope detection

For OOK-1/2/4 with heterodyne architecture with IF envelope detection, some sources ([7A-1] [7A-2] [7A-3] [7A-4] [7A-5] [7A-6] [7A-7] [7A-8]) provided analysis, with relative power consumption and noise figure summarized in in Table 7.1.1a-2.

- The relative power consumption for ON state is in the range of 0.1~4.

- The noise figure is in the range of 9~15 dB.

Table 7.1.1a-2 Relative power consumption and noise figure for OOK-1/2/4 with heterodyne architecture with IF envelope detection

| Source reference | [7A-1] | [7A-2] | [7A-3] | [7A-4] | [7A-5] | [7A-6] |
| --- | --- | --- | --- | --- | --- | --- |
| Power consumption (ON state) | 0.1 for single-branch, 0.01 for each additional branch | 0.5 | 0.1~1 | 0.1 | 0.1~1 | 1~4 |
| Noise figure (dB) | 15 | 10~15 | [9-15] | 15 | 12 | 12~15 |

##### 7.1.1a.1.3	Homodyne/zero-IF architecture with baseband envelope detection

For OOK-1/2/4 with homodyne/zero-IF architecture with baseband envelope detection, some sources ([7A-1] [7A-2] [7A-3] [7A-4] [7A-5] [7A-6] [7A-7] [7A-8] [7A-9] [7A-10]) provided analysis, with relative power consumption and noise figure summarized in in Table 7.1.1a-3.

- The relative power consumption for ON state is in the range of 0.05~4.

- The noise figure is in the range of 10~16 dB.

Table 7.1.1a-3 Relative power consumption and noise figure for OOK-1/2/4 with homodyne/zero-IF architecture with baseband envelope detection

| Source reference | [7A-1] | [7A-2] | [7A-3] | [7A-4] | [7A-5] | [7A-6] | [7A-7] | [7A-8] | [7A-9] | [7A-10] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Power consumption (ON state) | 0.09 for single-branch, 0.01 or 0.02 for each additional branch | 0.5 | 0.1~1 | 0.1 | 0.05~ 0.5 | 0.5~1 | 0.1~0.5 | 4 | ~1 | 0.1~0.5 |
| Noise figure (dB) | 15 | 10~15 | [10-16] | 15 | 12 | 15 | 12~15 | 15 | ~15 | 12 |

#### 7.1.1a.2	FSK

##### 7.1.1a.2.1	Parallel RF envelope detection

For FSK with parallel RF envelope detection, some sources ([7A-1] [7A-4]) provided analysis, with relative power consumption and noise figure summarized in Table 7.1.1a-4.

Table 7.1.1a-4 Relative power consumption and noise figure for FSK with parallel RF envelope detection

| Source reference | [7A-1] | [7A-4] |
| --- | --- | --- |
| Power consumption (ON state) | 0.05 for single-branch, 0.01 for each additional branch | 0.01 |
| Noise figure (dB) | 20 | 20 |

##### 7.1.1a.2.2	Parallel heterodyne architecture

For FSK with parallel heterodyne architecture, some sources ([7A-1] [7A-2] [7A-3] [7A-4]) provided analysis, with relative power consumption and noise figure summarized in Table 7.1.1a-5.

- The relative power consumption for ON state is in the range of 0.1~1.

- The noise figure is in the range of 9~15 dB.

Table 7.1.1a-5 Relative power consumption and noise figure for FSK with parallel heterodyne architecture

| Source reference | [7A-1] | [7A-2] | [7A-3] | [7A-4] |
| --- | --- | --- | --- | --- |
| Power consumption (ON state) | 0.1 for single-branch, 0.01 for each additional branch | 0.5 | 0.1~1 | 0.1 |
| Noise figure (dB) | 15 | 10~15 | [9~15] | 15 |

##### 7.1.1a.2.3	Parallel homodyne architecture

For FSK with parallel homodyne architecture, some sources ([7A-1] [7A-2] [7A-3] [7A-4]) provided analysis, with relative power consumption and noise figure summarized in Table 7.1.1a-6.

- The relative power consumption for ON state is in the range of 0.1~1.

- The noise figure is in the range of 10~16 dB.

Table 7.1.1a-6 Relative power consumption and noise figure for FSK with parallel homodyne architecture

| Source reference | [7A-1] | [7A-2] | [7A-3] | [7A-4] |
| --- | --- | --- | --- | --- |
| Power consumption (ON state) | 0.09 for single-branch, 0.01 or 0.02 for each additional branch | 0.5 | 0.1~1 | 0.1 |
| Noise figure (dB) | 15 | 10~15 | [10~16] | 15 |

##### 7.1.1a.2.4	Heterodyne architecture with frequency to amplitude conversion

For FSK with heterodyne architecture with frequency to amplitude conversion, some sources ([7A-1] [7A-3]) provided analysis, with relative power consumption and noise figure summarized in Table 7.1.1a-7.

Table 7.1.1a-7 Relative power consumption and noise figure for FSK with heterodyne architecture with frequency to amplitude conversion

| Source reference | [7A-1] (for SC-FSK) | [7A-3] |
| --- | --- | --- |
| Power consumption (ON state) | 0.1 | 1 |
| Noise figure (dB) | 15 | TBD |

##### 7.1.1a.2.5	Homodyne architecture with frequency to amplitude conversion

For FSK with homodyne architecture with frequency to amplitude conversion, 1 source ([7A-1]) estimated that for SC-FSK, the relative power consumption for ON state is 0.1, and the noise figure is 15dB.

#### 7.1.1a.3	OFDMA-based signal

For OFDMA-based signal, companies have considered sequence-based signal, where the LP WUR performs either time-domain correlation without FFT or frequency-domain correlation after FFT.

##### 7.1.1a.3.1	Time-domain correlation (without FFT)

For OFDM-based signal with time-domain correlation, some sources ([7A-1] [7A-3] [7A-5] [7A-6] [7A-7] [7A-8] [7A-9]) provided analysis, with relative power consumption and noise figure summarized in Table 7.1.1a-8.

- The relative power consumption for ON state is in the range of 0.15~30.

- The noise figure is in the range of 7~25 dB.

Table 7.1.1a-8 Relative power consumption and noise figure for OFDM-based signal with time-domain correlation

| Source reference | [7A-1] | [7A-3] | [7A-5] | [7A-6] | [7A-7] | [7A-8] | [7A-9] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Power consumption (ON state) | 0.15~0.2 | 10 | 10~20 | 10~30 | 1~5 | 10~20 | ~5 |
| Noise figure (dB) | 15 | 9.5 | 9.5 or 12 | 9 | 7~10 | 9 | 15~25 |

##### 7.1.1a.3.2	Frequency-domain correlation (with FFT)

For OFDM-based signal with frequency-domain correlation, some sources ([7A-2] [7A-3] [7A-5] [7A-7] [7A-10]) provided analysis, with relative power consumption and noise figure summarized in Table 7.1.1a-9.

- The relative power consumption for ON state is in the range of 1~30.

- The noise figure is in the range of 7~12 dB.

Table 7.1.1a-9 Relative power consumption and noise figure for OFDM-based signal with frequency-domain correlation

| Source reference | [7A-2] | [7A-3] | [7A-5] | [7A-7] | [7A-10] | [7A-10] |
| --- | --- | --- | --- | --- | --- | --- |
| Power consumption (ON state) | 10 | 30 | 20~30 | 1~5 | 10 | 4 |
| Noise figure (dB) | 7~12 | 7 | 9.5 or 12 | 7~10 | 9 | 12 |

#### 7.1.1a.4	References

[7A-1]	3GPP R1-2306549: “Discussion on architecture of LP-WUS receiver”, Huawei, HiSilicon.

[7A-2]	3GPP R1-2306692: " Discussion on LP-WUS receiver architectures", InterDigital, Inc.

[7A-3]	3GPP R1-2308472: "Remaining issues on low power wake-up receiver architecture", vivo.

[7A-4]	3GPP R1-2307063: "Low-power WUS receiver architectures and its performance", CATT.

[7A-5]	3GPP R1-2307143: "LP-WUS receiver architectures", ZTE, Sanechips.

[7A-6]	3GPP R1-2307700: "Receiver architecture for LP-WUS", Samsung.

[7A-7]	3GPP R1-2307990: "Low power WUS receiver architectures", Ericsson.

[7A-8]	3GPP R1-2307949: "Receiver architecture for LP-WUS", Qualcomm Incorporated.

[7A-9]	3GPP R1-2307300: "On low power wake-up receiver architectures", Apple, Inc.

[7A-10]	3GPP R1-2308068: "Low power WUS receiver architectures", MediaTek, Inc.

### 7.1.2	[RAN4 studies of the receiver]

Editor’s note: RAN4 related conclusions

## 7.2	LP-WUS design and L1 procedure

Editor’s note: The following SI Objective related to LP-WUS design and L1 procedure is included in this section.

- Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4]

- Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals [RAN2, RAN1]

- When evaluating and/or comparing link performance of MC-ASK, MC-FSK, and CP-OFDMA waveforms of LP-WUS at least

  - raw information bit-size

  - Alt 1:

    - average EPRE within the [time]/frequency resources used for LP-WUS (including any guard bands)

    - time/frequency resources used for LP-WUS (including any guard bands)

  - Alt 2:

    - average EPRE within the [time]/frequency resources used for LP-WUS (including any guard bands)

  - FFS: false alarm probability/rate

  - FFS: misdetection probability/rate

are kept [comparable or fixed].

- Study at least

  - impact of timing error

  - impact of frequency error

  - impact of phase noise and I/Q imbalance, if applicable

  - impact of ADC resolution and sampling rate

  - impact of interference

  - impact of delay spread

  - impact of doppler spread

- Companies to report

  - how they modelled SINR

    - SNR is calculated as average EPRE divided by power of noise [and interference].

  - whether and how power pooling across and within MR OFDMA symbols is used.

  - FFS: PAPR applicable to LP-WUS

  - time/frequency resources (including any guard bands) for the scheme

  - false alarm probability/rate and misdetection probability/rate

  - receiver architecture type and its relative power consumption

- When comparing waveforms of LP-WUS, consider the impact to gNB for each of the waveform generation schemes. Consider whether there is impact to PAPR and a need for additional hardware for WUS.

### 7.2.1	Signal design

#### 7.2.1.1	Waveform

- Study generation and link performance of multi-carrier (MC)-ASK (including OOK) waveform

  - study techniques to generate waveform by modulating sub-carriers of CP-OFDM symbol, consider up to M bits transmitted per OFDM symbol, where M is FFS.

    - Note that above does not preclude DFT-S-OFDMA

- Study generation and link performance of multi-carrier (MC)-FSK waveforms

  - study techniques to generate waveform by modulating sub-carriers of CP-OFDM symbol symbol, consider up to M bits transmitted per OFDM symbol, where M is FFS.

- Study link performance of OFDMA-based signals/channels considering at least the existing signal/channel structure (e.g. CSI-RS, SSS)

  - Other signal/channel structures are not precluded

For MC-ASK or MC-FSK waveform generation, SCS of a CP-OFDM symbol used for LP-WUS generation can be the same as SCS used for other NR transmissions in CP-OFDM symbol overlapping in time with, study whether SCS can be different, also study

- FDM/TDM multiplexing with other NR transmissions

- link performance

- impact to legacy UEs

- impact on gNB

- For waveform generation the following observations are made

  - Flat spectrum in frequency domain provides robustness against frequency selective fading compared to concentrated energy in frequency domain.

    - for OOK-4, sequence before DFT/LS with variation in phase via such as ZC, M-sequence or QAM sequence can achieve more flattened spectrum.

  - Sequences(s) used in LP-WUS symbol generation with different pulse shape or spectral shape may have different performance.

  - Knowledge of sequence(s) used in LP-WUS waveform generation may improve performance for at least a receiver with I/Q branches

- Further discuss the following potential observations for waveform generation:

  - When DFT is employed in OOK-4 (M>=2), -1/1 alternation in time or frequency shift in frequency domain may be needed to match CP-OFDM generation.

  - Pre-storing of the generated frequency domain samples at gNB may reduce complexity of waveform generation at gNB with memory requirement depending on number of possible combination. This may be up to gNB implementation.

  - Quantization of generated waveform in frequency domain to existing constellation (e.g. 64QAM) has low impact on performance and reduces complexity. This may be up to gNB implementation.

  - Repetition of a sequence(s) used in LP-WUS generation in frequency can be used to improve diversity for MC-OOK and robustness against frequency offsets for MC-FSK.

##### MC-ASK waveform

For MC-ASK waveform generation, where K is size of iFFT of CP-OFDMA, N is number of SCs used by LP-WUS including potential guard-bands, study further

- Option OOK-1: Single-bit in 1 OFDM symbol, SCs of LP-WUS are

  - OOK=1 means all SCs are modulated

  - OOK=0 means all SCs are zero power (from base-band point of view)

- Option OOK-2: Parallel M-bit OOK in frequency domain,

  - N SCs of LP-WUS is further separated into M segments (M=2 in Figure) possibly with guard-bands in-between and/or around

  - OOK=1 means all SCs in segment are modulated

  - OOK=0 means all SCs in segment are zero power (from base-band point of view)

  - OOK-2 can be received using the agreed receiver architectures for OOK with parallel envelope detection.

- Option OOK-3: Multi-tone single-bit OOK

  - N SCs of LP-WUS is separated into L segments (L=2 on Figure) without guard-bands in-between segment, but possibly around

  - OOK=1 means 1 sub-carrier (known by UE) of each segment is modulated, rest of SC is zero power (from base-band point of view)

  - OOK=0 means all SCs in all segments are zero power (from base-band point of view)

  - receiver architecture refers to section 7.1.1-F

- Option OOK-4: Transform M-bit OOK in time domain

  - N SCs of OOK-1 are generated by a transformation (DFT/Least square)

    - N’ samples are generated from M-bits

    - signal modification may or may NOT be used

    - truncation or other additional modification may or may NOT be used, if not used, N is the same as N’

  - N’ can be the same as K

- FFS modulated SCs are e.g. QAM symbols, sequences or other signals

  - Companies to report their assumptions

- potential guard-band SCs are zero power (from base-band point of view)

- optionally, 2 additional segments, one always modulated and one always zero power (from baseband point of view) can be transmitted

- Other options are not precluded (e.g. OOK-1 with multiple bits in one OFDM symbol)

Study further methods to modulate input signal of the DFT/Least-Square block for OOK-4, and methods to modulate input signal of N SCs for other MC-ASK/FSK schemes

- study methods with respect to

  - improving frequency diversity by flattening the spectrum, frequency repetition and frequency hopping

  - impact to dynamic range of RE power in frequency domain

  - FFS: impact to PAPR of generated time domain modulated MC-ASK/FSK symbol

  - improving robustness to timing error necessary spectrum adjustment for compatibility with CP-OFDM generation

##### FSK waveform

For M-bit MC-FSK generation study further the following options

- Option FSK-1: N SCs of LP-WUS are separated to M pairs of segments with potential guard-bands in-between and around.

  - segment comprises one sub-carrier or multiple contiguous SCs

  - in a pair of segments one segment is modulated, other segment is zero power (from base-band point of view)

- Option FSK-2: N SCs of LP-WUS are separated to 2^M segments with potential guard-bands in-between and around.

  - segment comprises one sub-carrier or multiple contiguous SCs

  - one segment from 2^M segments is modulated, other segments of SCs are zero power (from baseband point of view)

- M >0

- N >1

- Study how to generate segment in time domain, e.g. OOK-1 or OOK-4

- Other options are not precluded.

In FSK2-envelope-IF waveform generation, the N SCs of LP-WUS can be used to generate 2^M segments at the envelope of the LP-WUR’s received signal in baseband where each segment comprises one or more tones.

##### OFDM waveform

#### 7.2.1.2	Bandwidth and location

For the purpose of study, the BW of one LP-WUS is not greater than X (FFS X is 5 or 20) MHz for FR1, study further

- whether BW of LP-WUS is configurable (implicitly or explicitly)

- size of guard band [FFS: within or outside of BW X], if any

- whether there is different X for Idle, Connected, Inactive modes

FFS: Whether FR2 is included in the scope of LP-WUS SI

- From RAN1 perspective, LP-WUS and signals/channels used by MR can be within the same FR1 band.

  - At least LP-WUS and signals/channels by MR can be on the same carrier in the band

- Study further

  - Whether LP-WUS and signals/channels used by MR can be different carriers in the band

  - Details on the LP-WUS location within a carrier

  - Band can be different than band of signals/channels used by MR

  - LP-WUS association with BWP

  - LP-WUS can be configurable within guard-band of a band (like NB-IoT)

- Study RRC connected mode LP-WUS BW, whether same as IDLE/Inactive mode or different

At least for IDLE/Inactive mode, at least one BW-size <=5MHz is recommended to be supported for FR1

- Other BW sizes are not precluded

  - if additional BW-size(s) are recommended to be supported, BW-size can be up to 20MHz

- LP-WUS bandwidth size (including guard-bands) is assumed to be an integer number of PRBs

From RAN1 perspective, for multiplexing with other NR signals and channels, it is beneficial if LP-WUS can be flexibly configured within a carrier.

#### 7.2.1.3	LP-WUS content

- For IDLE/INACTIVE mode study at least following candidates for content of LP-WUS

  - information on which user(s) is/are targeted by the LP-WUS

    - e.g. UE-group, -subgroup or -ID

  - FFS: cell information

  - FFS: SI change and ETWS/CMAS information, tracking area information, and RAN area information

- For CONNECTED mode, study at least following candidates for content of LP-WUS

  - information on which user(s) is/are targeted by the LP-WUS

    - e.g UE-group, -subgroup or -ID

  - indication to wake-up to PDCCH monitoring.

- Other information candidates are not precluded

- Study pros and cons of including above information to LP-WUS.

- Note: the information may be explicitly or implicitly indicated.

- Study further following alternatives to carry the LP-WUS information using:

  - Alt 1: by sequence(s) detection/selection

    - FFS sequence type

  - Alt 2: by encoded bits

    - FFS: what type of encoding scheme

    - FFS: with or without other bits (e.g. CRC/FCS)

  - Other alternatives are not precluded

- Study whether LP-WUS information needs to be preceded by known one or more sequence(s).

<Editor’s Note: Any conclusion for LP-SS structure>

### 7.2.2	L1 procedures

#### 7.2.2.1	RRM measurements

Study potential measurement metric used for RRM measurements performed by LP-WUR.

- examples of measurement metric are signal quality, signal power, detection rate of LP-WUS/synch signal

- companies to report assumption of signal used for measurements

For at least RRM serving cell measurement performed by LP-WUR based on reference signal(s), RAN1 identified at least the following metrics for further study and evaluation (including feasibility, complexity, power consumption, etc)

- LP-RSSI or Energy detection: linear average of total received power over a RSSI resource.

    - FFS RSSI resource.

- LP-RSRP: linear average of received power of resource of reference signal(s) or signal(s) parts.

    - FFS resource of reference signal(s) or signal(s) parts

- LP-SINR = LP-RSRP/(power of interference and noise)

    - FFS how to define “power of interference and noise”

- LP-RSRQ= [N x] LP-RSRP/LP-RSSI, where N is the factor of resource size difference for evaluation LP-RSRP and LP-RSSI.

- Accounting AGC accuracy, ADC of at least 4 bits is required.

- Note: Reference signal for performing measurements can be e.g. SSB (PSS/SSS/PBCH DMRS), LP-WUS-waveform sequence, LP-SS

- Note: The definition of metrics could be further refined based on future study

RRC IDLE/INACTIVE mode

For a UE support LP-WUR in IDLE/INACTIVE mode,

- Study how to reduce UE power consumption due to existing RRM measurement requirements at least for mobility support,

  - study feasibility of RRM measurements performed by LP-WUR, at least for serving/camping cell, based on signals detected by LP-WUR

    - FFS: measurement metric

    - FFS: whether and how to identify cell/ tracking area

    - FFS: need for neighbouring cells

    - FFS: need for relaxation of existing RRM measurement requirements (for UE)

- For Idle/Inactive mode, study offloading of RRM measurements of serving cell to LP-WUR under certain conditions, if any, and relaxation of serving/neighboring cell RRM measurements in MR considering

- Periodic reference signal(s) is/are used for LR measurements.

- FFS: reference signal(s) to measure, e.g. PSS/SSS/PBCH DMRS, LP-WUS waveform sequence, LP-SS

- FFS: periodicity, content

- MR performs measurements

- Alt2: with relaxed periodicity if RRM measurement in MR is relaxed.

    - FFS: Condition for relaxation if any

    - Can apply for both neighboring and serving cell

- Alt3: only when reference signal(s) based measurements by LP-WUR satisfy certain condition(s), e.g. are below threshold.

    - FFS threshold.

    - Above MR measurement under certain conditions can apply for both neighboring and serving cell

    - Potentially with relaxation methods for MR neighboring cell measurement

- Other alternatives are not precluded

- FFS: Feasibility of RRM measurements of neighbor cells by LP-WUR

The followings are observed,

- At least for LP-WUR that cannot receive existing PSS/SSS, periodic LP-SS signal is beneficial for the following functionality.

  - RRM measurements by LP-WUR, if supported

  - at least coarse time synchronization of LP-WUR.

  - at least coarse frequency synchronization of LP-WUR.

- Additional periodic LP-SS system overhead depends on LP-SS periodicity, system BW, # of beams, and resource required to fulfil the target functionality, etc. Periodic signal if used for coarse synchronization may reduce overhead of signal preceding LP-WUS, if any. LP-SS can be designed to be common among UE groups (cell-specific) and such further reduce system overhead.

- For LP-WUR that can receive existing PSS/SSS potentially assisted by PBCH DMRS/TRS for synchronization, existing PSS/SSS potentially assisted by PBCH DMRS/TRS may be used for above functionality.

- Periodic LP-SS coverage should be equal to or better than that of LP-WUS.

- For fine time and frequency synchronization, a signal (e.g. preamble) preceding or part of LP-WUS may be used.

RRC CONNECTED mode

Additional descriptions for RRM RRC CONNECTED mode can be found in 7.2.2.6.

#### 7.2.2.2	Synchronization of LP-WUR

Study synchronisation signal used by LP-WUR, if needed, based on

- Option 1: aperiodic signal transmitted as part of LP-WUS

  - FFS: Whether the signal can additionally be transmitted separately from LP-WUS

- Option 2: periodic signal transmitted separately from LP-WUS

- Option 3: Option1 + Option2

Additional description of synchronization of LP-WUR can be found in 7.2.2.1.

#### 7.2.2.3	LP-WUS monitoring

Study further pros and cons of the following monitoring behaviours of LP-WUR

- Option1: Duty cycle, corresponds to LP-WUR switches between ON/OFF states

- Option2: Continuous monitoring, corresponds to LP-WUR is ON all the time

#### 7.2.2.4	Activation/Deactivation procedures of LP-WUS

- Study RRC connected mode LP-WUS activation/deactivation procedures.

- For Idle/Inactive mode, following options for activation and deactivation of LP-WUS monitoring by LP-WUR for a UE can be considered for study

- Alt 1a:

  - gNB transmits legacy paging indication and LP-WUS

  - UE activation and/or deactivation of LP-WUS WUS monitoring is up to UE implementation.

  - This behavior may apply based on channel condition, e.g. when coverage is sufficient/insufficient.

- Alt 1b:

  - gNB transmits legacy paging indication and LP-WUS

  - UE activation and/or deactivation of LP-WUS monitoring is based on preconfigured criteria

  - This behavior may apply based on channel condition, e.g. when coverage is sufficient/insufficient.

- Alt 2:

  - activation and/or deactivation of LP-WUS monitoring in a cell is based on signalling.

- Paging misdetection performance shall not be impacted.

- In RRC CONNECTED mode, LP-WUS monitoring can be activated/deactivated by at least one or more of

- by gNB RRC signaling, with or without UE assistance.

- by gNB L1/L2 LP-WUS activation/deactivation signaling, with or without UE assistance.

- based on pre-configured condition(s), such as timer.

- LP-WUS monitoring by UE is known to gNB, study whether it could be transparent to gNB.

- other options are not precluded.

#### 7.2.2.5	Coverage

- Study techniques/mechanisms to enhance coverage performance of LP-WUS

- Study potential gains available as well as drawback(s) of the technique(s)/mechanisms(s), e.g. system overhead, increased complexity network energy consumption etc…

- Study potential issues and corresponding solutions for the case when LP-WUS coverage is insufficient

  - At least study fallback mechanisms where the Main Radio switches to legacy operation in case the channel condition of LP-WUS is not sufficient, e.g. below threshold.

- Study the following techniques/mechanisms to enhance coverage performance of LP-WUS

  - low complex channel coding

    - FEC

    - spreading code in time domain

    - time domain repetition

    - with combining before or after ED

    - time-domain interleaving

    - Note: Also Manchester coding can be considered as channel code

  - non-contiguous transmission in the frequency domain

  - frequency domain repetition

  - frequency-hopping

  - power-boosting

  - transmit diversity

  - study whether any above techniques could be transparent to UE.

#### 7.2.2.6	Additional L1 procedures for RRC CONNECTED mode

- Study RRC connected mode LP-WUS functionality/purpose/procedures

- In RRC connected, study the relationship between LP-WUS and legacy UE power saving techniques.

- In RRC CONNECTED mode, study benefit of LP-WUS over existing Rel-15, R16, and R17 power saving techniques for following functionalities:

  - LP-WUS with similar functionality as R16 DCP.

  - LP-WUS activates/resumes PDCCH monitoring when LP-WUS is received.

    - interaction with legacy power saving techniques, if any

  - other functionalities are not precluded

  - for evaluation

    - companies to report

      - assumption on MR sleep state when LP-WUR is monitoring LP-WUS

        - deep sleep,

        - light sleep,

        - micro sleep

      - how to activate/deactivate LP-WUS monitoring and deactivate/activate PDCCH monitoring

      - LP-WUS waveform

- For RRC connected mode, the following is assumed for LP-WUS study in RAN1

  - RLM/BFD/CSI are performed by UE Main Radio (MR)

  - RRM measurements are performed by UE Main Radio (MR)

  - Ultra-deep sleep state is not allowed for MR.

- Study additional support of RRM measurement by LP-WUR for RRC connected mode

#### 7.2.2.7	Additional L1 procedures for RRC IDLE/INACTIVE modes

#### 7.2.2.8	Others

## 7.3	Higher-layer aspects

Editor’s note: The following SI Objective related to higher layer aspects is included in this section.

- Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals [RAN2, RAN1]

# 8	Evaluation Results

Editor’s note: The following SI Objective is included in this section.

- Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms and their coverage availability, as well as latency impact. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1]

  - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

## 8.1	Power and latency evaluation

### 8.1.1	RRC IDLE/INACTIVE mode

The following ‘Effective per UE paging arrival rate (i.e., Reff)’ is defined as (without taking FAR into account)

- Per UE paging probability RE if LP-WUS is per UE paging

- Per group paging probability RG = 1 – (1 – RE)N, if LP-WUS is per group paging (N is the number of UEs in the group)

The following names are used,

- Reff 		Effective per UE paging arrival rate

- T			the period for complete an on-and-off cycle for LP-WUS duty-cycled monitoring,

- D 		the active time for monitoring LP-WUS every cycle for LP-WUS duty-cycled monitoring

- R 		LP-WUR duty cycle ratio = D/T

- PWURON		LP-WUR on state power

- PWUROFF		LP-WUR off state power

#### 8.1.1.1	Duty cycled LP-WUS

In this section, duty cycled LP-WUS is studied and compared to I-DRX cases. In addition, the following is assumed,

- no RRM measurement performed by MR or LR, or RRM measurement offloaded to LR or MR RRM relaxed at least 8 times of I-DRX cycle

- MR in ultra-deep sleep if no traffic is arrival

- MR ramp-up time, transition energy = 400ms, 15000unit,

- I-DRX cycle = 1.28s

- FAR <=1% (0%, 0.001%, 0.1%, 1%)

##### 8.1.1.1.1	Collection of the results

###### 8.1.1.1.1.1	LR (PWURON, R, FAR) = (<10unit, <=1%, <=1%)

I-DRX cycle = 1.28s, LP-WUR on state power <10unit, LP-WUR duty cycle ratio <=1%,  FAR <=1% (0%, 0.001%, 0.1%, 1%)

Figure 8.1.1.1 - 1

Note1: The results combine different LP-WUR(ON, OFF) power values. And detailed simulated combinations can be seen in Table 8.1.1.3-1

Note2: [8A-12] assumes MR ramp-up time, transition energy is 100ms, 2000unit(not aligned with agreed assumption)

Note3: WUS monitoring duration is assumed as follows:

- [8A-16](2ms)/ [8A-11](2ms)/ [8A-17](1ms)/ [8A-9] (1ms)/ [8A-3] (1/1.5/2.5ms)/ [8A-12] (1.26ms)/ [8A-8] (1.28ms)/ [8A-7] (0.25) assume very short WUS monitoring duration.

- [8A-11](12.8ms)/ [8A-10](8ms)/ [8A-1] (10ms)/ [8A-5] (10ms)/ [8A-15] (6/9/10/12ms)/ [8A-8] (12.8ms)/ [8A-3] (6/10/13ms)/ [8A-4] (13ms)/ [8A-13] (10ms) assume long WUS monitoring duration.

Note4: FAR is assumed as follows:

- FAR = 0 is simulated by [8A-3], [8A-11], [8A-8], [8A-5]

- FAR = 0.001% is simulated by [8A-12, [8A-15]]

- FAR = 0.1% is simulated by [8A-3], [8A-4], [8A-17], [8A-11], [8A-13], [8A-16], [8A-8]

- FAR = 1% is simulated by [8A-3], [8A-4], [8A-17], [8A-11], [8A-13], [8A-1], [8A-10], [8A-16], [8A-8], [8A-7]

Note5: all the possible values for LP-WUR ON power < 10 are mixed together, further split is shown in section 8.1.1.3

###### 8.1.1.1.1.2	LR (PWURON, R, FAR) = (<10unit, (1%,5%], <=1%)

I-DRX cycle = 1.28s, LP-WUR on state power <10unit, 1%< LP-WUR duty cycle ratio <=5%,  FAR <=1% (0.001%, 0.1%, 1%)

Figure 8.1.1.1 - 2

Note1: The results combine different LP-WUR(ON, OFF) power values. And detailed simulated combinations can be seen in Table 8.1.1.3-1

Note2: FAR is assumed as follows:

- FAR = 0.001% is simulated by [8A-15]

- FAR = 0.1% is simulated by [8A-3]

- FAR = 1% is simulated by [8A-7]

Note3: all the possible values for LP-WUR ON power < 10 are mixed together, further split is shown in section 8.1.1.3

Note 4: Results with duty cycle ratio 5~30% are provided by [8A-3], [8A-5], [8A-8], [8A-11], and included in the spread sheet for power evaluation

###### 8.1.1.1.1.3	LR (PWURON, R, FAR)  = (>=10unit, <=1%, <=1%)

I-DRX cycle = 1.28s, LP-WUR on state power >=10unit, LP-WUR duty cycle ratio <=1%,  FAR <=1% (0%, 0.001%, 0.1%, 1%)

Figure 8.1.1.1 - 3

Note1: The results combines different LP-WUR(ON, OFF) power values. And detailed simulated combinations can be seen in Table 8.1.1.3-1

Note2: WUS monitoring duration is assumed as follows:

- [8A-7](0.25ms)/ [8A-16] (2ms)/ [8A-8] (1.28/2.56ms)/ [8A-3] (1/1.5/2.5ms)/ [8A-11] (2ms)/ [8A-9] (1ms)/ [8A-10] (2ms) assume very short WUS monitoring duration.

- [8A-15](6/9/10/12ms)/ [8A-8](6.4/12.8ms)/ [8A-3] (6/10ms)/ [8A-11] (12.8ms)/ [8A-1] (10ms)/ [8A-13] (10ms) assume long WUS monitoring duration.

Note3: FAR is assumed as follows:

- FAR = 0 is simulated by [8A-3], [8A-11], [8A-8]

- FAR = 0.001% is simulated by [8A-15]

- FAR = 0.1% is simulated by [8A-3], [8A-9], [8A-11], [8A-13], [8A-16], [8A-8]

- FAR = 1% is simulated by [8A-3], [8A-11], [8A-13], [8A-1], [8A-10], [8A-16], [8A-8], [8A-7]

Note4: all the possible values for LP-WUR ON power > 10 are mixed together, further split is shown in section 8.1.1.3

###### 8.1.1.1.1.3	Latency

I-DRX cycle = 1.28s, LP-WUR duty cycle ratio <=1%, effective per UE paging arrival rate <=8%

Figure 8.1.1.1 – 4 legacy PO

Note: [8A-5] assumes UE-specific indication and therefore latency is the time interval between data arrival at the gNB until the UE finish sync/re-sync for MR, while other companies assume latency is the time interval between data arrival at the gNB until the first PO that UE can monitor for paging message.

Figure 8.1.1.1 – 5 dynamic PO

Note: [8A-13] assumes LP-WUS duty cycle is 1280ms.

Note: [8A-8] assumes the effective monitoring periodicity for LP-WUS is 128ms.

Note: [8A-11] assumes LP-WUS duty cycle is 200ms

##### 8.1.1.1.2	Observations

For duty-cycled LP-WUS monitoring, the following observations are made with the assumption that

- no RRM measurement performed by MR or LR, or RRM measurement offloaded to LR or MR RRM relaxed at least 8 times of I-DRX cycle

- MR in ultra-deep sleep if no traffic is arrival

- MR ramp-up time, transition energy = 400ms, 15000unit,

- I-DRX cycle = 1.28s

- FAR <=1% (0%, 0.001%, 0.1%, 1%)

- effective per UE paging arrival rate <=8%

- For WUR ON power < 10unit (i.e., 0.01/0.05/0.1/0.5/1/2/4 unit) with no more than duty cycle ratio 1%

- Effective per UE paging arrival rate<=1%

    - Results in [8A-1], [8A-3], [8A-4], [8A-5], [8A-7], [8A-8], [8A-9], [8A-10], [8A-11], [8A-12], [8A-13], [8A-15], [8A-16], [8A-17] show that compared with i-DRX with or without PEI, LP-WUS provides mean power saving gain (average: 77%, range: 30%~97%)

- 1%<Effective per UE paging arrival rate <=8%

    - Results in [8A-1], [8A-3], [8A-5], [8A-8], [8A-10] show that compared with i-DRX with or without PEI, LP-WUS provides mean power saving gain (average: 69%, range: 53%~83%)

- For WUR ON power < 10unit (i.e., 0.01/0.05/0.1/0.5/1/2/4 unit) with duty cycle ratio between 1% and 5%

- Effective per UE paging arrival rate<=1%

    - Results in [8A-3], [8A-7], [8A-15] show that compared with i-DRX with or without PEI, LP-WUS provides mean power saving gain (average: 92%, range 87%~98%)

- 1%<Effective per UE paging arrival rate <=8%

    - Results in [8A-3] show that compared with i-DRX with or without PEI, LP-WUS provides mean power saving gain (81%).

- For WUR ON power>=10unit (i.e., 10/20/30 unit) with no more than duty cycle ratio 1%

- Effective per UE paging arrival rate <=1%

    - Results in [8A-1], [8A-3], [8A-7], [8A-8], [8A-9], [8A-10], [8A-11], [8A-13], [8A-15], [8A-16] show that compared with i-DRX with or without PEI, LP-WUS provides mean power saving gain (average: 62%, range: -13%~95%)

- 1%<Effective per UE paging arrival rate <=8%

    - Results in [8A-1], [8A-3], [8A-8], [8A-10], [8A-15] show that compared with i-DRX with or without PEI, LP-WUS provides mean power saving gain (average: 63%, range: 42%~81%)

- For latency

- Legacy PO

    - Results in [8A-1], [8A-4], [8A-5], [8A-7], [8A-8], [8A-11], [8A-15], [8A-17] show that compared with i-DRX with or without PEI mean paging latency (0.638~0.768) second, LP-WUS will result in mean paging latency (1.022~1.264) second, which increases mean paging latency (52%~98%)

    - Results in [8A-3], [8A-10], [8A-13], [8A-16] show that compared with i-DRX with or without PEI paging latency, LP-WUS will result in mean paging latency (1.053~1.909) second without baseline i-DRX latency.

- Dynamic PO

    - Results in [8A-8], [8A-11] show that compared with i-DRX with or without PEI mean paging latency (0.638~0.687) second, LP-WUS will result in mean paging latency (0.568~0.599) second, which decreases mean paging latency (6%~17%)

    - Results in [8A-13] show that compared with i-DRX with or without PEI paging latency, LP-WUS will result in mean paging latency (1.08) second without baseline i-DRX latency.

#### 8.1.1.2	Continuous monitoring results and results with duty cycle ratio = 50% LP-WUS

In this section, continuous monitoring of LP-WUS and LP-WUS monitoring with duty cycle ratio of 50% are studied and the following is assumed,

- Continuous monitoring results and results with duty cycle ratio = 50% LP-WUS comparing with I-DRX is as baseline

- FAR target is determined across a reference time duration T of multiple LP-WUS attempts/trials.

- no RRM measurement performed by MR or LR, or RRM measurement offloaded to LR or MR RRM relaxed at least 8 times of I-DRX cycle

- MR in ultra-deep sleep if no traffic is arrival

- MR ramp-up time, transition energy = 400ms, 15000unit

- I-DRX cycle = 1.28s

- FAR<=1% (0%, 0.001%, 0.1%, 1%)

Table 8.1.1.2 - 1 FAR assumptions of different sources

| Sources | (FAR_target, N, T) |
| --- | --- |
| [8A-2] | Not considered as FAR=0 |
| [8A-3] | (0.1%, 1, 5ms) |
| [8A-4] | N=1280, T=1.28s |
| [8A-5] | N=1500, T=1.28s |
| [8A-7] | (1%, 1, 1.28s) |
| [8A-8] | (0%, 1, 1.28s) / (1%, 1, 1.28s) |
| [8A-9] | (0.1%, 1280, 1.28s) |
| [8A-11] | (0.1%, 1, 1.28s) / (1%, 1, 1.28s) |
| [8A-13] | (0.1%, N=1~64, 1.28s) / (1%, N=1~64, 1.28s) |
| [8A-14] | (0, 1280, 1.28s) |
| [8A-15] | (0.001%, 1, 1ms) |
| [8A-16] | (0.1%, 1280, 1.28s) / (1%, 1280, 1.28s) |

##### 8.1.1.2.1	Collection of the results

###### 8.1.1.2.1.1	LR (PWURON, FAR)  = (<= 1unit, <=1%)

I-DRX cycle = 1.28s, T = I-DRX cycle, LP-WUR on state power <=1unit, FAR<=1% (0%, 0.001%, 0.1%, 1%)

Figure 8.1.1.2 - 1

Note1: The results tagged with [8A-2](1) are for LP-WUS UEs, and the results tagged with [8A-2](2) are for the system UEs which contain both LP-WUS UEs and legacy UEs who are unable to apply LP-WUS monitoring

Note2: For (FAR, N, T), [8A-3] assumes (T= 5ms, N=1), [8A-15] assumes (T=1ms, N=1), while other companies assume T = I-DRX cycle period, N value in (FAR, N, T) is summarized in table 8.1.1.2-1.

Note3: FAR is assumed as follows:

- FAR = 0 is simulated by [8A-2], [8A-5], [8A-11], [8A-8], [8A-14]

- FAR = 0.001% is simulated by [8A-15]

- FAR = 0.1% is simulated by [8A-3], [8A-4], [8A-5], [8A-17], [8A-9], [8A-11], [8A-13], [8A-16]

- FAR = 1% is simulated by [8A-4], [8A-5], [8A-17], [8A-11], [8A-13], [8A-7], [8A-16], [8A-8]

###### 8.1.1.2.1.2	LR (PWURON, FAR)  = ( > 1unit, <=1%)

I-DRX cycle = 1.28s, T = I-DRX, LP-WUR on state power >1unit, FAR<=1% (0% , 0.1%, 1%)

Figure 8.1.1.2 – 2

Note1 : FAR is assumed as follows:

- FAR = 0 is simulated by [8A-5, [8A-11], [8A-8], [8A-14]]

- FAR = 0.1% is simulated by [8A-4], [8A-17], [8A-9], [8A-11], [8A-16]

- FAR = 1% is simulated by [8A-4], [8A-17], [8A-11], [8A-16], [8A-8]

Note2: For (FAR, N, T), [8A-3] assumes (T= 5ms, N=1), [8A-15] assumes (T=1ms, N=1), while other companies assume T = I-DRX cycle period, N value in (FAR, N, T) is summarized in table 8.1.1.2-1.

###### 8.1.1.2.1.3	Latency

I-DRX cycle = 1.28s, T = I-DRX cycle, effective per UE paging arrival rate <=8%

Figure 8.1.1.2 – 3 legacy PO

Note: [8A-5] assumes UE-specific indication and therefore latency is the time interval between data arrival at the gNB until the UE finish sync/re-sync for MR, while other companies assume latency is the time interval between data arrival at the gNB until the first PO that UE can monitor for paging message.

Figure 8.1.1.2 – 4 dynamic PO

##### 8.1.1.2.2	Observations

For continuous and duty cycle ratio = 50% based LP-WUS monitoring, the following observations are made with the assumption that

- I-DRX cycle = 1.28s

- T = I-DRX cycle

- MR ramp-up time, transition energy = 400ms, 15000unit

- no RRM measurement performed by MR or LR, or RRM measurement offloaded to LR or MR RRM relaxed at least 8 times of I-DRX cycle

- MR in ultra-deep sleep.

- FAR<=1% (0%, 0.001%, 0.1%, 1%)

- effective per UE paging arrival rate <=8%

- For WUR ON power <=1unit

- Effective per UE paging arrival rate <=1%

- Results in [8A-2], [8A-4], [8A-5], [8A-7], [8A-8], [8A-9], [8A-11], [8A-13], [8A-14], [8A-15], [8A-16], [8A-17] show that compared with i-DRX with or without PEI, LP-WUS provides mean power saving gain (average: 61.5%, range: 21%~97%, which assumes FAR reference time T= 1280ms.

- Results in [8A-3] show that compared with i-DRX with or without PEI, LP-WUS provides mean power saving gain (-12%) which assumes FAR reference time T= 5ms.

- 1%<Effective per UE paging arrival rate <=8%

- Results in [8A-8], [8A-15] show that compared with i-DRX with or without PEI, LP-WUS provides mean power saving gain (average:58%, range: 45%~72%)

- For WUR ON power >1unit

- Effective per UE paging arrival rate <=1%

- Results in [8A-4], [8A-5], [8A-8], [8A-9], [8A-11], [8A-14], [8A-16], [8A-17] show that compared with i-DRX with or without PEI, LP-WUS provides mean power saving gain (average: -248.5%, range: -674%~-61%)

- 1%<Effective per UE paging arrival rate <=8%

- Results in [8A-8] show that compared with i-DRX with or without PEI, LP-WUS provides mean power saving gain (-778%).

- For latency (LP-WUS Continuous monitoring results and results with duty cycle ratio=50%)

- Legacy PO

- Results in [8A-4], [8A-7], [8A-8], [8A-11], [8A-14],[8A-15], [8A-17 show that compared with i-DRX with or without PEI mean paging latency (0.637~0.768) second, LP-WUS will result in mean paging latency (0.902~1.189) second, which increases mean paging latency (34%~91%)

- Results in [8A-5] show that compared with i-DRX with or without PEI mean paging latency (0.67) second, LP-WUS will result in mean paging latency (0.571) second, which decreases mean paging latency (15%) by one company. This company assumes UE-specific indication and therefore latency is the time interval between data arrival at the gNB until the UE finishes sync/re-sync for MR.

- Results in [8A-3], [8A-13], [8A-16] show that compared with i-DRX with or without PEI paging latency, LP-WUS will result in mean paging latency (1.053~1.382) second without baseline i-DRX latency.

- Dynamic PO

- Results in [8A-5], [8A-11] show that compared with i-DRX with or without PEI mean paging latency (0.637~0.67) second, LP-WUS will result in mean paging latency (0.42~0.5) second, which decreases mean paging latency (22%~37%)

- Results in [8A-13] show that compared with i-DRX with or without PEI paging latency, LP-WUS will result in mean paging latency (0.45) second without baseline i-DRX latency.

#### 8.1.1.3	Various LP-WUR relative power for duty-cycled monitoring

In this section, various relative (ON, OFF) power of LP-WUR is compared and I-DRX is as baseline. In addition, the following is assumed,

- no RRM measurement performed by MR or LR, or RRM measurement offloaded to LR or MR RRM relaxed at least 8 times of I-DRX cycle

- MR in ultra-deep sleep if no traffic is arrival

- MR ramp-up time, transition energy = 400ms, 15000unit

- I-DRX cycle = 1.28s

The table summarizes the simulated combinations of LP-WUR ON and OFF power. In each combination ‘X’ in ‘y(X)’ stands for the number of simulated cases.

Table 8.1.1.3 - 1 simulated combinations for LP-WUR (ON, OFF) power values

| ON Power value | 0.01 |  |  | 0.05 |  |  | 0.1 |  |  | 0.2 |  |  | 0.5 |  |  | 1 |  |  |  | 2 |  |  | 4 |  |  |  |  | 10 |  |  |  |  | 20 |  |  | 30 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OFF Power value | 0.001 | 0.02 | 0.1 | 0.001 | 0.02 | 0.1 | 0.001 | 0.02 | 0.1 | 0.001 | 0.02 | 0.1 | 0.001 | 0.02 | 0.1 | 0.001 | 0.01 | 0.02 | 0.1 | 0.001 | 0.02 | 0.1 | 0.001 | 0.01 | 0.02 | 0.05 | 0.1 | 0.001 | 0.01 | 0.02 | 0.05 | 0.1 | 0.001 | 0.02 | 0.1 | 0.001 | 0.02 | 0.1 |
| [8A-1] |  |  |  |  |  |  | Y |  |  |  |  |  |  |  |  | Y |  |  |  |  |  |  | Y |  | Y |  | Y | Y |  |  |  |  |  |  |  |  |  |  |
| [8A-3] |  |  |  |  |  |  |  |  |  |  |  |  | Y |  |  |  |  |  |  |  |  |  | Y | Y |  | Y |  | Y | Y |  | Y |  |  |  |  |  |  |  |
| [8A-4] | Y |  |  | Y |  |  | Y |  |  |  |  |  | Y |  |  | Y |  |  |  | Y |  |  | Y |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| [8A-5] |  |  |  |  |  |  | Y |  |  |  |  |  |  |  |  | Y |  |  |  |  |  |  | Y |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| [8A-17] | Y |  |  |  |  |  | Y |  |  |  |  |  | Y |  |  | Y |  |  |  |  |  |  | Y |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| [8A-7] |  |  |  |  |  |  |  | Y |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Y |  |  |  |  | Y |  |  |  |  |  |  |  |  |
| [8A-15] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Y |  |  |  |  |  | Y |  |  |  |  |  |  |  |  |  |
| [8A-9] | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y |  | Y | Y | Y | Y | Y | Y |  | Y |  | Y | Y |  | Y |  | Y | Y | Y | Y | Y | Y | Y |
| [8A-16] |  |  |  |  |  |  |  |  |  |  |  |  | Y | Y | Y | Y |  | Y | Y |  |  |  | Y |  | Y |  | Y | Y |  | Y |  | Y | Y | Y | Y | Y | Y | Y |
| [8A-8] | Y |  |  | Y |  |  | Y |  |  |  |  |  | Y | Y |  | Y |  |  |  | Y |  |  | Y |  |  |  |  |  |  |  |  |  | Y |  |  |  |  | Y |
| [8A-12] |  |  |  |  |  |  | Y |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| [8A-10] | Y |  |  |  |  |  |  |  |  |  |  |  | Y |  |  |  | Y | Y |  |  |  |  |  | Y | Y |  |  |  |  |  |  | Y |  |  |  |  |  | Y |
| [8A-11] | Y |  |  |  |  |  | Y |  |  |  |  |  |  |  |  | Y |  |  |  |  |  |  |  |  |  |  |  | Y |  | Y |  | Y |  | Y | Y |  |  |  |
| [8A-13] | Y |  |  |  |  |  |  |  |  |  |  |  | Y |  |  | Y |  |  |  |  |  |  |  |  |  |  |  |  |  | Y |  | Y |  |  |  |  | Y | Y |

##### 8.1.1.3.1	Collection of the results

###### 8.1.1.3.1.1	LR (FAR, Reff, R) = (<=0.1%, <=0.1%, <=1%)

I-DRX cycle = 1.28s, Effective per UE paging arrival rate <=0.1% , LP-WUR duty cycle ratio <=1%, FAR<=0.1% (0%, 0.001%, 0.1%)

Figure 8.1.1.3 - 1

Note1: [8A-12] assumes MR ramp-up time, transition energy is 100ms, 2000unit(not aligned with agreed assumption)

Note2: WUS monitoring duration is assumed as follows:

- [8A-7](0.25ms)/ [8A-16](2ms) /[8A-11](2ms)/ [8A-3] (1/1.5/2.5ms)/ [8A-17] (1ms)/ [8A-12] (1.26ms) assume very short WUS monitoring duration.

- [8A-3](6/10/13ms)/ [8A-11] (12.8ms)/ [8A-13] (10ms)/ [8A-4] (13/38ms) assume long WUS monitoring duration.

Note3: FAR is assumed as follows:

- FAR = 0 is simulated by [8A-3], [8A-11]

- FAR = 0.001% is simulated by [8A-12][8A-15]

- FAR = 0.1% is simulated by [8A-3], [8A-4], [8A-17], [8A-11], [8A-13], [8A-16]

###### 8.1.1.3.1.2	LR (FAR, Reff, R) = (<=1%, (0.1%,1%], <=1%)

I-DRX cycle = 1.28s, 0.1%< Effective per UE paging arrival rate <=1% , LP-WUR duty cycle ratio <=1%, FAR<=1% (0%, 0.001%, 0.1%, 1%)

Figure 8.1.1.3 - 2

Note1: [8A-12] assumes MR ramp-up time, transition energy is 100ms, 2000unit(not aligned with agreed assumption)

Note2: WUS monitoring duration is assumed as follows:

- [8A-7](0.25ms)/ [8A-3] (1/1.5/2.5ms)/ [8A-16] (2ms)/ [8A-10] (2ms) /[8A-11] (2ms)/ [8A-17] (1ms)/ [8A-8] (1.28ms)/ [8A-9] (1ms) assume very short WUS monitoring duration.

- [8A-8](12.8ms)/ [8A-3](6/10/13ms)/ [8A-11] (12.8ms)/ [8A-1](10ms)/ [8A-13](10ms)/ [8A-10] (8ms)/ [8A-4] (13/38ms)/ [8A-5] (10ms) assume long WUS monitoring duration.

Note3: FAR is assumed as follows:

- FAR = 0 is simulated by [8A-3], [8A-5], [8A-11], [8A-8]

- FAR = 0.001% is simulated by [8A-12], [8A-15]

- FAR = 0.1% is simulated by [8A-3], [8A-4], [8A-17], [8A-9], [8A-11], [8A-13], [8A-16], [8A-8]

- FAR = 1% is simulated by [8A-3], [8A-4], [8A-17], [8A-11], [8A-13], [8A-10], [8A-1], [8A-7], [8A-16], [8A-8]

###### 8.1.1.3.1.3	LR (FAR, Reff, R) = (<=0.1%, <=0.1%, (1%,10%])

I-DRX cycle = 1.28s, Effective per UE paging arrival rate <=0.1% , 1%<LP-WUR duty cycle ratio <=10%, FAR<=0.1%  (0%, 0.001%, 0.1%)

Figure 8.1.1.3 - 3

Note: FAR is assumed as follow:

- FAR = 0 is simulated by [8A-11]

- FAR = 0.001% is simulated by [8A-15]

- FAR = 0.1% is simulated by [8A-3], [8A-11]

###### 8.1.1.3.1.4	LR (FAR, Reff, R) = (<=1%, (0.1%,1%], (1%,10%] )

I-DRX cycle = 1.28s, 0.1%< Effective per UE paging arrival rate <=1%, 1%<LP-WUR duty cycle ratio <=10%, FAR <=1% (0%, 0.001%, 0.1%, 1%)

Figure 8.1.1.3 – 4

Note: FAR is assumed as follows:

- FAR = 0 is simulated by [8A-11], [8A-8]

- FAR = 0.001% is simulated by [8A-15]

- FAR = 0.1% is simulated by [8A-3], [8A-11]

- FAR = 1% is simulated by [8A-11], [8A-7]

##### 8.1.1.3.2	Observations

For various relative power of LP-WUR, the following observations are made with the assumption that

- I-DRX cycle = 1.28s

- MR ramp-up time, transition energy = 400ms, 15000unit

- no RRM measurement performed by MR or LR, or RRM measurement offloaded to LR or MR RRM relaxed at least 8 times of I-DRX cycle

- MR in ultra-deep sleep.

Observations:

- For (FAR, Effective per UE paging arrival rate, duty cycle ratio) = (<=0.1%, <=0.1%, <=1%)

- WUR ON power <=1unit

- Results in [8A-3], [8A-4], [8A-11], [8A-12], [8A-13], [8A-16], [8A-17] show that compared with i-DRX with or without PEI, LP-WUS provides UE mean power saving gain 57%~99% (average 80%), corresponding to 1%~43% (average 20%) of UE power consumption in legacy I-DRX operation

- 2unit<= WUR ON power <=4unit

- Results in [8A-3], [8A-4], [8A-15], [8A-16], [8A-17] show that compared with i-DRX with or without PEI, LP-WUS provides UE mean power saving gain 55%~97% (average 83%), corresponding to 3%~45% (average 17%) of UE power consumption in legacy I-DRX operation.

- 5unit<= WUR ON power <=10unit

- Results in [8A-3], [8A-11], [8A-13], [8A-16] show that compared with i-DRX with or without PEI, LP-WUS provides UE mean power saving gain 29%~95% (average 66%), corresponding to 5%~71% (average 34%) of UE power consumption in legacy I-DRX operation.

- 20unit<= WUR ON power <=30unit

- Results in [8A-11], [8A-13], [8A-16] show that compared with i-DRX with or without PEI, LP-WUS provides UE mean power saving gain -79%~86% (average 16%), corresponding to 14%~179% (average 84%) of UE power consumption in legacy I-DRX operation.

- For (FAR, Effective per UE paging arrival rate, duty cycle ratio) = (<=1%, (0.1%,1%], <=1%)

- WUR ON power <=1unit

- Results in [8A-1], [8A-3], [8A-4], [8A-5], [8A-8], [8A-9], [8A-10], [8A-11], [8A-12], [8A-13], [8A-16], [8A-17] show that compared with i-DRX with or without PEI, LP-WUS provides UE mean power saving gain 31%~91% (average 74%), corresponding to 9%~69% (average 26%) of UE power consumption in legacy I-DRX operation.

- 2unit<= WUR ON power <=4unit

- Results in [8A-1], [8A-3], [8A-4], [8A-5], [8A-7], [8A-8], [8A-9], [8A-10], [8A-15], [8A-17] show that compared with i-DRX with or without PEI, LP-WUS provides UE mean power saving gain 30%~94% (average 72%), corresponding to 6%~70% (average 28%) of UE power consumption in legacy I-DRX operation.

- 5unit<= WUR ON power <=10unit

- Results in [8A-1], [8A-3], [8A-7], [8A-9], [8A-10], [8A-11], [8A-13], [8A-15], [8A-16] show that compared with i-DRX with or without PEI, LP-WUS provides UE mean power saving gain 40%~92% (average 69%), corresponding to 8%~60% (average 31%) of UE power consumption in legacy I-DRX operation.

- 20unit<= WUR ON power <=30unit

- Results in [8A-8], [8A-9], [8A-10], [8A-11], [8A-13], [8A-16] show that compared with i-DRX with or without PEI, LP-WUS provides UE mean power saving gain -58%~63% (average 25%), corresponding to 37%~158% (average 75%) of UE power consumption in legacy I-DRX operation.

- For (FAR, Effective per UE paging arrival rate, duty cycle ratio) = (<=0.1%, <=0.1%, (1%,10%])

- WUR ON power <=1unit

- Results in [8A-3], [8A-11] show that compared with i-DRX with or without PEI, LP-WUS provides UE mean power saving gain 84%~97% (average 90%), corresponding to 3%~16% (average 10%) of UE power consumption in legacy I-DRX operation.

- 2unit<= WUR ON power <=4unit

- Results in [8A-15] show that compared with i-DRX with or without PEI, LP-WUS provides UE mean power saving gain 85%, corresponding to 15% of UE power consumption in legacy I-DRX operation.

- 5unit<= WUR ON power <=10unit

- Results in [8A-11] show that compared with i-DRX with or without PEI, LP-WUS provides UE mean power saving gain 24%, corresponding to 76% of UE power consumption in legacy I-DRX operation.

- 20unit<= WUR ON power <=30unit

- Results in [8A-11] show that compared with i-DRX with or without PEI, LP-WUS provides UE mean power saving gain -34%, , corresponding to 134% of UE power consumption in legacy I-DRX operation

- For (FAR, Effective per UE paging arrival rate, duty cycle ratio) = (<=1%, (0.1%,1%], (1%,10%])

- WUR ON power <=1unit

- Results in [8A-3], [8A-7], [8A-8], [8A-11] show that compared with i-DRX with or without PEI, LP-WUS provides UE mean power saving gain 68%~94% (average 84%), corresponding to 6%~32% (average 16%) of UE power consumption in legacy I-DRX operation.

- 2unit<= WUR ON power <=4unit

- Results in [8A-8], [8A-15] show that compared with i-DRX with or without PEI, LP-WUS provides UE mean power saving gain 81%~87%(average 84%), corresponding to 13%~19% (average 16%) of UE power consumption in legacy I-DRX operation.

- 5unit<= WUR ON power <=10unit

- Results in [8A-11], [8A-15] show that compared with i-DRX with or without PEI, LP-WUS provides UE mean power saving gain 1%~82%(average 41%), corresponding to 18%~99% (average 59%) of UE power consumption in legacy I-DRX operation.

- 20unit<= WUR ON power <=30unit

- Results in [8A-8], [8A-11] show that compared with i-DRX with or without PEI, LP-WUS provides UE mean power saving gain -63%~31%(average -16%), corresponding to 69%~163% (average 116%) of UE power consumption in legacy I-DRX operation.

#### 8.1.1.4	Compared with eDRX

In this section, Duty cycled LP-WUS is compared to eDRX and eDRX is as baseline.

For eDRX, the effective paging rate is defined per Y_REF, where Y_REF =1.28s.

##### 8.1.1.4.1	Collection of the results

###### 8.1.1.4.1.1	LP-WUS monitoring within PTW and legacy PO determination

- LP-WUR duty cycle ratio <=1%,

- no RRM measurement performed by MR or LR, or RRM measurement offloaded to LR or MR RRM relaxed at least 8 times of I-DRX cycle

- MR in ultra-deep sleep

Figure 8.1.1.4 - 1

Figure 8.1.1.4 – 2

Note：[8A-3] assumes eDRX cycle is 51.2/593.92s.

###### 8.1.1.4.1.2	LP-WUS monitoring not restricted in PTW and dynamic PO determination

- LP-WUR duty cycle ratio <=1%,

- no RRM measurement performed by MR or LR, or RRM measurement offloaded to LR or MR RRM relaxed at least 8 times of I-DRX cycle

- MR in ultra-deep sleep

Figure 8.1.1.4 - 3

Figure 8.1.1.4 - 4

##### 8.1.1.4.2	Observations

For duty-cycled LP-WUS, comparing with eDRX, the following observations are made with the assumption that

- LP-WUR duty cycle ratio <=1%

- no RRM measurement performed by MR or LR, or RRM measurement offloaded to LR or MR RRM relaxed at least 8 times of I-DRX cycle

- MR in ultra-deep sleep.

Observations:

- LP-WUS monitoring within PTW and legacy PO determination

- Effective per UE paging arrival rate <=1%

- Results in [8A-3], [8A-4], [8A-8], [8A-12], [8A-16] show that compared with eDRX, LP-WUS provide mean power saving gain (average: 67.5%, range: 28%~92%)

- Results in [8A-4], [8A-8] show that compared with eDRX mean paging latency (16.697~25.92) second, LP-WUS will result in mean paging latency (19.863~31.24) second, which increases mean paging latency (19%~21%)

- Results in [8A-3], [8A-16] show that compared with eDRX, LP-WUS will result in mean paging latency (42.679~161.73) second without baseline eDRX latency.

- 1%<Effective per UE paging arrival rate <=8%

- Results in [8A-8] show that compared with eDRX, LP-WUS provides mean power saving gain (75%).

- Results in [8A-8] show that compared with eDRX mean paging latency (16.927) second, LP-WUS will result in mean paging latency (16.953) second, which increases mean paging latency (0.2%)

- LP-WUS monitoring is not restricted in PTW and dynamic PO determination

- Effective per UE paging arrival rate <=1%

- Results in [8A-7], [8A-8], [8A-11] show that compared with eDRX, LP-WUS provides mean power saving gain (average: 22%, range: 9%~58%)

- Results in [8A-7], [8A-8], [8A-11] show that compared with eDRX mean paging latency (16.927~28.192) second, LP-WUS will result in mean paging latency (0.566~1.035) second, which decreases mean paging latency (96%~98%) if LP-WUS monitoring and the corresponding paging monitoring after MR wake-up is performed not restricted within existing PTW of eDRX.

- 1%<Effective per UE paging arrival rate <=8%

- Results in [8A-8] show that compared with eDRX, LP-WUS provides mean power saving gain (-15%).

- Results in [8A-8] show that compared with eDRX mean paging latency (16.927) second, LP-WUS will result in mean paging latency (0.566) second, which decreases mean paging latency (97%).

#### 8.1.1.5	RRM for LP-WUR duty-cycled monitoring

In this section, measurement performed by MR only and MR RRM relaxed X times and RRM offload to LR are studied for duty-cycled LP-WUS monitoring. In addition, the following is assumed,

- I-DRX cycle = 1.28s

- MR in ultra-deep sleep

- Effective per UE paging arrival rate <=1%

- LP-WUR duty cycle ratio <=1%

- MR ramp-up time, transition energy = 400ms, 15000unit

- FAR<=1%  (0%, 0.001%, 0.1%, 1%)

##### 8.1.1.5.1	Collection of the results

###### 8.1.1.5.1.1	LR (FAR, Reff, R) = (<=1%, <=1%, <=1%)

I-DRX cycle = 1.28s, Effective per UE paging arrival rate <=1% , LP-WUR duty cycle ratio <=1%, baseline: I-DRX, FAR<=1% (0%, 0.001%, 0.1%, 1%)

Figure 8.1.1.5-1

Note1: [8A-15] assumes that the MR is in deep sleep when there is no any MR RRM relaxation, while other companies assume MR ultra-deep sleep

Note2: [8A-12] assumes MR ramp-up time, transition energy is 100ms, 2000unit(not aligned with agreed assumption)

Note3: FAR is assumed as follows:

- FAR = 0 is simulated by [8A-3], [8A-11], [8A-8], [8A-5]

- FAR = 0.001% is simulated by [8A-12], [8A-15]

- FAR = 0.1% is simulated by [8A-3], [8A-4], [8A-17], [8A-11], [8A-13], [8A-16], [8A-8]

- FAR = 1% is simulated by [8A-3], [8A-17], [8A-1], [8A-10], [8A-16], [8A-8], [8A-7], [8A-4]

Note 4: Different LR measurement periodicity are used by companies

##### 8.1.1.5.2	Observations

For RRM with duty-cycled LP-WUS monitoring, the following observations are made with the assumption that

- I-DRX cycle = 1.28s

- MR in ultra-deep sleep

- Effective per UE paging arrival rate <=1%

- LP-WUR duty cycle ratio <=1%

- MR ramp-up time, transition energy = 400ms, 15000unit

- FAR<=1% (0%, 0.001%, 0.1%, 1%)

Observations:

- For (Effective per UE paging arrival rate, LP-WUR on state power, FAR) = (<=1%, <=1unit, <=1%)

Compared with i-DRX, LP-WUS operation with

- No MR RRM relaxed

- Results in [8A-3], [8A-8], [8A-11], [8A-12], [8A-16] show that compared with i-DRX with or without PEI, LP-WUS provide mean power saving gain (average: -211%, range: -624%~-5%), assuming MR enters ultra-deep sleep

- Results in [8A-15] show that compared with i-DRX with or without PEI, LP-WUS provides power saving gain 21%, assuming MR enters deep sleep

- MR relaxed < 8 times

- Results in [8A-1], [8A-3], [8A-8], [8A-9], [8A-15], [8A-16] show that compared with i-DRX with or without PEI, LP-WUS provide mean power saving gain (average: 4%, range: -18%~50%)

- 8 times<= MR relaxed <=16 times

- Results in [8A-1], [8A-3], [8A-4], [8A-8], [8A-9], [8A-11], [8A-12], [8A-13], [8A-15] show that compared with i-DRX with or without PEI, LP-WUS provide mean power saving gain (average: 40%, range: -18%~60%)

- MR relaxed > 16 times

- Results in [8A-1], [8A-3], [8A-4], [8A-8], [8A-11], [8A-13], [8A-15], [8A-16], [8A-17] show that compared with i-DRX with or without PEI, LP-WUS provide mean power saving gain (average: 60%, range: 22%~90%)

- MR offload RRM to LR

- Results in [8A-3], [8A-4], [8A-5], [8A-7], [8A-8], [8A-10], [8A-11], [8A-15], [8A-16], [8A-17] show that compared with i-DRX with or without PEI, LP-WUS provide mean power saving gain (average: 83%, range: 76%~94%)

#### 8.1.1.6	RRM for LP-WUR continuously monitoring

In this section, measurement performed by MR only and MR RRM relaxed X times and RRM offload to LR are studied for continuous LP-WUS monitoring. In addition, the following is assumed,

- I-DRX cycle = 1.28s

- MR in ultra-deep sleep

- LP-WUR on state power <= 1unit

- Effective per UE paging arrival rate <=1%

- MR ramp-up time, transition energy = 400ms, 15000unit

- FAR<=1% (0%, 0.001%, 0.1%, 1%)

##### 8.1.1.6.1	Collection of the results

###### 8.1.1.6.1.1	LR (Reff, PWURON, FAR) = (<=1unit, <=1%, <=1%)

I-DRX cycle = 1.28s, Effective per UE paging arrival rate <=1% , LP-WUR on state power <=1unit, baseline: I-DRX, FAR<=1% (0%, 0.001%, 0.1%, 1%)

Figure 8.1.1.6-1

Note1: [8A-15] assumes that the MR is in deep sleep when there is no any MR RRM relaxation while other companies assume ultra-deep sleep.

Note2: FAR is assumed as follow:

- FAR = 0 is simulated by [8A-11], [8A-8], [8A-5], [8A-14]

- FAR = 0.001% is simulated by [8A-15]

- FAR = 0.1% is simulated by [8A-4], [8A-17], [8A-11], [8A-13], [8A-16], [8A-9], [8A-5]

- FAR = 1% is simulated by [8A-4], [8A-17], [8A-11], [8A-16], [8A-8], [8A-7], [8A-5]

Note3: For (FAR, N, T), [8A-3] assums (T= 5ms, N=1), [8A-15] assumes (T=1ms, N=1), while other companies assume T = I-DRX cycle period, N value in (FAR, N, T) is summarized in table 8.1.1.2-1.

##### 8.1.1.6.2	Observations

For RRM with continuous LP-WUS monitoring, the following observations are made with the assumption that

- I-DRX cycle = 1.28s

- MR in ultra-deep sleep

- LP-WUR on state power <= 1unit

- Effective per UE paging arrival rate <=1%

- MR ramp-up time, transition energy = 400ms, 15000unit

- FAR<=1% (0%, 0.001%, 0.1%, 1%)

Observations

- For (Effective per UE paging arrival rate, ON power, FAR) = (<=1%, <=1unit, <=1%)

Compared with i-DRX, LP-WUS operation with

- No MR RRM relaxed

- Results in [8A-11], [8A-16], show that compared with i-DRX with or without PEI, LP-WUS provides mean power saving gain (average: -325.5%, range: -644%~-7%) assuming MR enters ultra-deep sleep

- Results in [8A-15] show that compared with i-DRX with or without PEI, LP-WUS provides power saving gain 20%, assuming MR enters deep sleep

- MR relaxed < 8 times

- Results in [8A-8], [8A-15], [8A-16] show that compared with i-DRX with or without PEI, LP-WUS provides mean power saving gain (average: 22%, range: 7%~49%)

- 8 times<= MR relaxed <=16 times

- Results in [8A-4], [8A-5], [8A-9], [8A-11], [8A-13], [8A-15] show that compared with i-DRX with or without PEI, LP-WUS provide mean power saving gain (average: 32%, range: 2%~61%)

- MR relaxed > 16 times

- Results in [8A-4], [8A-5], [8A-8], [8A-11], [8A-13], [8A-15], [8A-16], [8A-17] show that compared with i-DRX with or without PEI, LP-WUS provide mean power saving gain (average: 57%, range: 37%~80%)

- MR offload RRM to LR

- Results in [8A-4], [8A-5], [8A-7], [8A-8], [8A-11], [8A-14], [8A-15], [8A-16], [8A-17] show that compared with i-DRX with or without PEI, LP-WUS provide mean power saving gain (average: 74%, range: 52%~87%)

#### 8.1.1.7	Impact by various MR sync/re-sync assumptions

##### 8.1.1.7.1	Collection of the results

###### 8.1.1.7.1.1	LR (Reff, R, PWURON, FAR) = (<=1%, <=1%,<= 4unit, <=1%)

I-DRX cycle = 1.28s,

FAR <= 1%, (0%, 0.1%, 1%)

Effective per UE paging arrival rate <=1% ,

duty cycle ratio<= 1%,

no RRM measurement performed by MR or LR, or RRM measurement offloaded to LR or MR RRM relaxed at least 8 times of I-DRX cycle

baseline: I-DRX

Figure 8.1.1.7-1

Note: For [8A-4] results, different SNR assumptions for the baseline scheme contribute to the difference in power saving gain between (sync/re-sync energy <= 1000) and (1000< sync/re-sync energy <= 3000).

Note: FAR is assumed as follow:

- FAR = 0 is simulated by [8A-11], [8A-8], [8A-5]

- FAR = 0.1% is simulated by [8A-4], [8A-17], [8A-11], [8A-16], [8A-9], [8A-8]

- FAR = 1% is simulated by [8A-4], [8A-17], [8A-11], [8A-10], [8A-16], [8A-8], [8A-7]

###### 8.1.1.7.1.2	LR (Reff, R, PWURON, FAR) = (<=1%, <=1%, >= 10unit, <=1%)

DRX cycle = 1.28s,

FAR <= 1%, (0%, 0.1%, 1%)

Effective per UE paging arrival rate <=1% ,

duty cycle ratio<= 1%,

no RRM measurement performed by MR or LR, or RRM measurement offloaded to LR or MR RRM relaxed at least 8 times of I-DRX cycle

baseline: I-DRX

Figure 8.1.1.7-2

Note: FAR is assumed as follow:

- FAR = 0 is simulated by [8A-8], [8A-11]

- FAR = 0.1% is simulated by [8A-11], [8A-16], [8A-9], [8A-8]

- FAR = 1% is simulated by [8A-11], [8A-16], [8A-8], [8A-7], [8A-10]

##### 8.1.1.7.2	Observations

For various MR sync/re-sync evaluation, the following observations are made with the assumption that

- I-DRX cycle = 1.28s

- LP-WUR duty cycle ratio <=1%

- Effective per UE paging arrival rate <=1%

- no RRM measurement performed by MR or LR, or RRM measurement offloaded to LR or MR RRM relaxed at least 8 times of I-DRX cycle

- MR in ultra-deep sleep.

- FAR<=1% (0%, 0.1%, 1%)

Observations:

- For (Effective per UE paging arrival rate, duty cycle ratio, ON power, FAR) = (<=1%, <=1%,<=4unit, <=1%)

- 1000unit<MR sync/re-sync energy consumption <=3000unit

- Results in [8A-4], [8A-5], [8A-7], [8A-8] show that compared with i-DRX with or without PEI, LP-WUS provides mean power saving gain (66%~87%)

- MR sync/re-sync energy consumption > 3000unit

- Results in [8A-4], [8A-9], [8A-10], [8A-11], [8A-17] show that compared with i-DRX with or without PEI, LP-WUS provides mean power saving gain (41%~83%)

- For (Effective per UE paging arrival rate, duty cycle ratio, ON power, FAR) = (<=1%, <=1%,>=10unit, <=1%)

- 1000unit<MR sync/re-sync energy consumption <=3000unit

- Results in [8A-7], [8A-8] show that compared with i-DRX with or without PEI, LP-WUS provides mean power saving gain (46%~79%)

- MR sync/re-sync energy consumption > 3000unit

- Results in [8A-9], [8A-10], [8A-11] show that compared with i-DRX with or without PEI, LP-WUS provides mean power saving gain (41%~70%)

#### 8.1.1.8	Impact by various MR ramp-up time/transition energy assumptions and sleep state

##### 8.1.1.8.1	Collection of the results

###### 8.1.1.8.1.1	LR (Reff, R, PWURON, FAR) = (<=1%, <=1%,<=4unit, <=1%)

DRX cycle = 1.28s,

Effective per UE paging arrival rate <=1% ,

duty cycle ratio<=1%,

no RRM measurement performed by MR or LR, or RRM measurement offloaded to LR or MR RRM relaxed at least 8 times of I-DRX cycle

baseline: I-DRX

FAR<=1% (0%, 0.001%, 0.1%, 1%)

Figure 8.1.1.8-1

Note1: 450 is associated with MR deep sleep and 15000/40000 is associated with MR ultra-deep sleep

Note2: FAR is assumed as follows:

- FAR = 0 is simulated by [8A-3], [8A-11,] [8A-5], [8A-8]

- FAR = 0.001% is simulated by [8A-15]

- FAR = 0.1% is simulated by [8A-3], [8A-17], [8A-11], [8A-16], [8A-9], [8A-13], [8A-8]

- FAR = 1% is simulated by [8A-3], [8A-17], [8A-11], [8A-16], [8A-13], [8A-8], [8A-10], [8A-1], [8A-7]

###### 8.1.1.8.1.2	LR (Reff, R, PWURON, FAR) = (<=1%, <=1%,>=10unit, <=1%)

DRX cycle = 1.28s,

Effective per UE paging arrival rate <=1%

duty cycle ratio<=1%

no RRM measurement performed by MR(including measurement offloaded to LR) or MR RRM relaxed at least 8 times of I-DRX cycle,

baseline: I-DRX

FAR<=1% (0%, 0.001%, 0.1%, 1%)

Figure 8.1.1.8-2

Note1: 450 is associated with MR deep sleep and 15000/40000 is associated with MR ultra-deep sleep

Note2: FAR is assumed as follows:

- FAR = 0 is simulated by [8A-3], [8A-11], [8A-8]

- FAR = 0.001% is simulated by [8A-15]

- FAR = 0.1% is simulated by [8A-3], [8A-9], [8A-11], [8A-16], [8A-13], [8A-8]

- FAR = 1% is simulated by [8A-3], [8A-11], [8A-16], [8A-13], [8A-8], [8A-10], [8A-1], [8A-7]

##### 8.1.1.8.2	Observations

For various MR ramp-up time/transition energy evaluation, the following observations are made with the assumption that

- I-DRX cycle = 1.28s

- Effective per UE paging arrival rate <=1%

- FAR<=1% (0%, 0.001%, 0.1%, 1%)

- LP-WUR duty cycle ratio<=1%

- no RRM measurement performed by MR or LR, or RRM measurement offloaded to LR or MR RRM relaxed at least 8 times of I-DRX cycle

- MR in ultra-deep sleep when MR transition energy is 15000/40000unit

- MR in deep sleep when MR transition energy is 450unit.

Observations:

- For (Effective per UE paging arrival rate, duty cycle ratio, ON power, FAR) = (<=1%, <=1%,<=4unit, <=1%)

- (Baseline for MR ultra-deep sleep) MR ramp-up time, energy consumption = 400ms, 15000unit

- Results in [8A-1], [8A-3], [8A-4], [8A-5], [8A-7], [8A-8], [8A-9], [8A-10], [8A-11], [8A-13], [8A-15], [8A-16], [8A-17] show that compared with i-DRX with or without PEI, LP-WUS provide mean power saving gain (average: 76%, range: 43%~92%)

- (Optional for MR ultra-deep sleep) MR ramp-up time, energy consumption = 800ms, 40000unit

- Results in [8A-1], [8A-13], [8A-17] show that compared with i-DRX with or without PEI, LP-WUS provides mean power saving gain (average: 44%, range: 14%~82%)

- (MR deep sleep) MR ramp-up time, energy consumption = 10~20ms, 450unit

- Results in [8A-1], [8A-3], [8A-7], [8A-10], [8A-11] show that compared with i-DRX with or without PEI, LP-WUS provides mean power saving gain (average: 49%, range: 34%~64%)

- For (Effective per UE paging arrival rate, duty cycle ratio, ON power, FAR) = (<=1%, <=1%,>=10unit, <=1%)

- (Baseline for MR ultra-deep sleep) MR ramp-up time, energy consumption = 400ms, 15000unit

- Results in [8A-1], [8A-3], [8A-7], [8A-8], [8A-9], [8A-10], [8A-11], [8A-13], [8A-15], [8A-16] show that compared with i-DRX with or without PEI, LP-WUS provide mean power saving gain (average: 60%, range: -11%~93%)

- (Optional for MR ultra-deep sleep) MR ramp-up time, energy consumption = 800ms, 40000unit

- Results in [8A-1], [8A-13] show that compared with i-DRX with or without PEI, LP-WUS provides mean power saving gain (average: 49%, range: 25%~73%)

- (MR deep sleep) MR ramp-up time, energy consumption = 10~20ms, 450unit

- Results in [8A-1], [8A-7], [8A-10], [8A-11] show that compared with i-DRX with or without PEI, LP-WUS provides mean power saving gain (average: 35%, range: 29%~51%)

### 8.1.2	RRC CONNECTED mode

#### 8.1.2.1	XR traffic model

In this section, the usage of LP-WUS for XR traffic is studied.

The following schemes are compared, and the description of the schemes are as follows,

- Always on

  - i.e., no DRX, UE is always available for gNB scheduling

- C-DRX

  - Including long and short DRX

- R17 PDCCH Scheme

  - i.e., R17 PDCCH skipping and/or R17 SSSG switching, with or without C-DRX configuration

- R18 enhanced C-DRX

  - i.e., aligning C-DRX cycle with non-integer traffic periodicity

- LP-WUS

  - LP-WUS triggers the resumption of MR PDCCH monitoring from micro/light sleep e.g., via starting a PDCCH monitoring timer.

Note: MR based RRM measurements were not explicitly modelled for power saving evaluation.

##### 8.1.2.1.1	Collection of the results

###### 8.1.2.1.1.1	DL only traffic

Figure 8.1.2.1 - 1

Note: the values shown in the figure are averaged gain.

Note: For XR, all companies assume 60FPS DL only traffic. [8A-11], [8A-13] and [8A-7] assume the data rate of DL traffic is 30Mbps, [8A-14] assumes the data rate of DL traffic is 45Mbps.

Note: For the case of R17 PDCCH as the baseline, [8A-13] evaluation included C-DRX, while [8A-14], [8A-7]results did not include C-DRX configuration

Figure 8.1.2.1 - 2

Figure 8.1.2.1 -3

Note: the results with the setting that number of deployed UE per cell exceeds the upper bound of system capacity are excluded.

Note: high load and low load are reported by companies, or the low load is for the case that number of UE per cell is no more than half of the capacity upper bound and high load is for the others.

Note: the values shown in the figure are averaged gain.

###### 8.1.2.1.1.2	DL + UL traffics

Figure 8.1.2.1 - 4

Note: [8A-11] provides both the results for single DL flow or two DL flows (DL video flow: 60FPS, 30Mbps and 10ms PDB; DL audio flow: 10ms periodicity, 0.756Mbps and 30ms PDB) and single UL flow (UL pose flow: 16.67ms periodicity and 10ms PDB).

Note: [8A-3] provide the results for DL two flows (DL video flow: 60FPS and 10ms PDB; DL audio flow: 10ms periodicity and 30ms PDB) and single UL flow (UL pose flow:4ms periodicity and 10ms PDB).

Note: the values shown in the figure are averaged gain.

Figure 8.1.2.1 - 5

Figure 8.1.2.1 - 6

Note: the results with the setting that number of deployed UE exceed the upper bound of system capacity are excluded.

Note: high load and low load are reported by companies, or the low load is for the case that Number of UE per cell is no more than half of the capacity upper bound and high load is for the others.

Note: the values shown in the figure are averaged gain.

##### 8.1.2.1.2	Observations

<Editor’s Note: the average value, the min and max value in the range is calculated across all companies’ results>

For DL only XR traffic in low load, compared with legacy schemes,

- For power saving gain, LP-WUS can provide

  - Results in [8A-3], [8A-11], [8A-14] show that power saving gain (average: 11.2%, range:2%~23%), assuming MR enters micro sleep during LP-WUS monitoring.

  - Results in [8A-11] show that power saving gain (average: 20%, range:14%~26%), assuming MR enters light sleep during LP-WUS monitoring.

  - the legacy schemes evaluated by companies included R17 PDCCH skipping, SSSG switching, R18 enhanced DRX, R15 short CDRX.

- For satisfied UE rate, LP-WUS can provide

  - Results in [8A-3], [8A-11], [8A-14] show that the relative impact of satisfied UE rate (average: +0.3%, range: -1%~+2%), assuming MR enters micro sleep during LP-WUS monitoring.

  - Results in [8A-11] show that the relative impact of satisfied UE rate -1%, assuming MR enters light sleep during LP-WUS monitoring.

  - the legacy schemes evaluated by companies included R17 PDCCH skipping, SSSG switching, R18 enhanced DRX, R15 short CDRX.

For DL only XR traffic in high load, compared with legacy schemes,

- For power saving gain, LP-WUS can provide

  - Results in [8A-3] [8A-11] [8A-13] [8A-7] show that power saving gain (average: 11.6%, range: 2%~19%), assuming MR enters micro sleep during LP-WUS monitoring.

  - Results in [8A-11] show that power saving gain (average: 18%, range: 12%~24%), assuming MR enters light sleep during LP-WUS monitoring.

  - the legacy schemes evaluated by companies included R17 PDCCH skipping, SSSG switching, R18 enhanced DRX, R15 short CDRX.

- For satisfied UE rate, LP-WUS can provide

  - Results in [8A-3] [8A-11] [8A-13] [8A-7] show that the relative impact of satisfied UE rate (average: +0.96%, range: -3.8%~+5%), assuming MR enters micro sleep during LP-WUS monitoring.

  - Results in [8A-11] show that the relative impact of satisfied UE rate (average: -15.2%, range: -15.9%~-14.4%), assuming MR enters light sleep during LP-WUS monitoring.

  - the legacy schemes evaluated by companies included R17 PDCCH skipping, SSSG switching, R18 enhanced DRX, R15 short CDRX.

For DL + UL XR traffics in low load, compared with legacy schemes,

- For power saving gain, LP-WUS can provide

  - Results in [8A-3] [8A-11] show that power saving gain (average: 6.5%, range: 0%~12%), assuming MR enters micro sleep during LP-WUS monitoring;

  - Results in [8A-11] show that power saving gain (average: 19%, range: 18%~20%), assuming MR enters light sleep during LP-WUS monitoring;

  - the legacy schemes evaluated by companies included R17 PDCCH skipping, SSSG switching, R18 enhanced DRX, R15 short CDRX.

- For satisfied UE rate, LP-WUS can provide

  - Results in [8A-3] [8A-11] show that the relative impact of satisfied UE rate (average: +0.25%, range: 0%~+1%), assuming MR enters micro sleep during LP-WUS monitoring;

  - Results in [8A-11] show that the relative impact of satisfied UE rate (average: -3.5%, range: -6%~-1%), assuming MR enters light sleep during LP-WUS monitoring;

  - the legacy schemes evaluated by companies included R17 PDCCH skipping, SSSG switching, R18 enhanced DRX, R15 short CDRX.

For DL + UL XR traffics in high load, compared with legacy schemes,

- For power saving gain, LP-WUS can provide

  - Results in [8A-3] [8A-11] show that power saving gain (average: 5.5%, range: 0%~10%), assuming MR enters micro sleep during LP-WUS monitoring;

  - Results in [8A-11] show that power saving gain (average: 16.5%, range: 16%~17%), assuming MR enters light sleep during LP-WUS monitoring;

  - the legacy schemes evaluated by companies included R17 PDCCH skipping, SSSG switching, R18 enhanced DRX, R15 short CDRX.

- For satisfied UE rate, LP-WUS can provide

  - Results in [8A-3] [8A-11] show that the relative impact of satisfied UE rate (average: -0.5%, range: -3%~+2%), assuming MR enters micro sleep during LP-WUS monitoring;

  - Results in [8A-11] show that the relative impact of satisfied UE rate (average: -15.5%, range: -16%~-15%), assuming MR enters light sleep during LP-WUS monitoring;

  - the legacy schemes evaluated by companies included R17 PDCCH skipping, SSSG switching, R18 enhanced DRX, R15 short CDRX.

#### 8.1.2.2	FTP 3 and IM traffic model

In this section, the usage of LP-WUS for FTP3 and IM traffic is studied.

For the evaluated FTP3 traffic, packet size: 0.5Mbyte and average inter-arrival time: 200ms are assumed by companies, unless otherwise noted.

For the evaluated IM traffic, packet size: 0.1 Mbyte and average inter-arrival time: 2seconds are assumed by companies, unless otherwise noted.

The following schemes are compared, and the description of the schemes are as follows,

- Always on

  - i.e., no DRX , UE is always available for gNB scheduling

- C-DRX

- DCP

  - R16 wake-up signal for RRC CONNECTED UE

- R17 PDCCH Scheme

  - i.e., R17 PDCCH skipping and/or R17 SSSG switching

- LP-WUS scheme 1

  - LP-WUS to trigger the resumption of MR PDCCH monitoring from micro/light/deep sleep.

Note: LP-WUS scheme1 does not require C-DRX configuration.

- LP-WUS scheme 2

  - 1) LP-WUS to replace DCP, i.e. LP-WUS to trigger the start of the drx_onDurationTimer.

  - 2) LP-WUS to replace DCP outside DRX active time, and LP-WUS to trigger the resumption of MR PDCCH monitoring during DRX active time (including drx_onDurationTimer and drx-inactivityTimer). (evaluated by [8A-1])

Note: LP-WUS scheme 2 can only be used with C-DRX configuration.

Note: MR based RRM measurements were not explicitly modelled for power saving evaluation.

##### 8.1.2.2.1	Collection of the results

###### 8.1.2.2.1.1	LP-WUS Scheme 1

Figure 8.1.2.2 – 1a (deep sleep)

Figure 8.1.2.2 – 1b (light sleep)

Figure 8.1.2.2 – 1c (micro sleep)

Note: [8A-7] assume duty cycled LP-WUS monitoring with 40 or 80ms duty cycle. And the others assume continuous LP-WUS monitoring.

Note: [8A-14] assumes the packet size of FTP3 traffic is 0.1Mbyte.

Note: the values shown in the figure are average gain.

Figure 8.1.2.2 - 2

Note: [8A-7] assume duty cycled LP-WUS monitoring with 40 or 80ms duty cycle. And the others assume continuous LP-WUS monitoring.

Note: the values shown in the figure are average gain

Figure 8.1.2.2 – 3a(deep sleep)

Figure 8.1.2.2 – 3b(light sleep)

Figure 8.1.2.2 – 3c (micro sleep)

###### 8.1.2.2.1.2	LP-WUS Scheme 2

For LP-WUS used as to replace DCP, the power saving and UPT impact are summarized in figure 8.1.2.2-4 and 8.1.2.2.5, respectively.

Figure 8.1.2.2 - 4

Figure 8.1.2.2 – 5

Note:

- [8A-2] , [8A-8] , [8A-11] assume LP-WUS is used to replace DCP and LP-WUR ON power value no more than 1 unit,

- [8A-15] assumes LP-WUS is used to replace DCP and LP-WUR ON power value is 0.1/4/10units.

- 200ms/400ms/800ms/1.6s/5s/10s average interval times for FTP3 traffic are evaluated by [8A-2].

- The assumed ramp up time of MR from deep/light sleep is 10ms/3ms

In [8A-1]’s evaluation, in addition to DCP replacement, LP-WUS is also used to trigger the resumption of MR PDCCH during DRX active time(including drx_onDurationTimer and drx-inactivityTimer) and LP-WUR ON power value is 1/4/10 unit. The results are shown in figure 8.1.2.2-6 and 8.1.2.2-7.

Figure 8.1.2.2 – 6

Figure 8.1.2.2 – 7

Note: [8A-1] assumes the ramp up time of MR from deep/light sleep is 20ms/6ms

##### 8.1.2.2.2	Observations

<Editor’s Note: the average value, the min and max value in the range is calculated across all companies’ results>

For the usage of LP-WUS is to trigger the resumption of PDCCH monitoring (i.e., LP-WUS scheme1),

- For FTP3 traffic, when WUR ON power setting is NO more than 4units,

- LP-WUS compared with CDRX or CDRX+DCP provide

  - Results in [8A-11] [8A-7] [8A-13] [8A-14] show that power saving gain 24%~86% (average: 61.3%) when MR is allowed to enter in deep-sleep state during LP-WUS monitoring

  - Results in [8A-11] [8A-13] [8A-14] show that power saving gain -3%~41% (average: 23.75%) when MR is allowed to enter in light-sleep state during LP-WUS monitoring

  - Results in [8A-11] [8A-14] show that power saving gain -74%~-25% (average: -36%) when MR is allowed to enter in micro-sleep state during LP-WUS monitoring

- LP-WUS compared with R17 PDCCH + CDRX+ DCP provide

  - Results in [8A-5] [8A-13] show that power saving gain 44%~62% (average: 53%) when MR is allowed to enter in deep-sleep state during LP-WUS monitoring

  - Results in [8A-5] [8A-13] show that power saving gain -11%~18% (average: 3.5%) when MR is allowed to enter in light-sleep state during LP-WUS monitoring

  - Results in [8A-5] shows that power saving gain -26% when MR is allowed to enter in micro-sleep state during LP-WUS monitoring

- LP-WUS compared with alwayson provide

  - Results in [8A-11] shows that power saving gain 40%~53% (average: 46.5%) when MR is allowed to enter in micro-sleep state during LP-WUS monitoring

- For FTP3 traffic, when WUR ON power setting is equal to 10 units,

- LP-WUS compared with CDRX or CDRX+ DCP provide

  - Results in [8A-7] [8A-11] [8A-13] show that power saving gain 24%~58% (average: 40.25) when MR is allowed to enter in deep-sleep state during LP-WUS monitoring

  - Results in [8A-11] [8A-13] show that power saving gain -33%~12% (average: -10.5%) when MR is allowed to enter in light-sleep state during LP-WUS monitoring

  - Results in [8A-11] shows that power saving gain -114% when MR is allowed to enter in micro-sleep state during LP-WUS monitoring

- LP-WUS compared with R17 PDCCH + CDRX+ DCP provide

  - Results in [8A-13] shows that power saving gain 33% when MR is allowed to enter in deep-sleep state during LP-WUS monitoring

  - Results in [8A-13] shows that power saving gain-40% when MR is allowed to enter in light-sleep state during LP-WUS monitoring

- For IM traffic, when WUR ON power setting is NO more than 4units,

- LP-WUS compared with CDRX or CDRX+DCP provide

  - Results in [8A-7] shows that power saving gain 69% when MR is allowed to enter in deep-sleep state during LP-WUS monitoring

- LP-WUS compared with R17 PDCCH + CDRX+ DCP provide

  - Results in [8A-5] shows that power saving gain 79% when MR is allowed to enter in deep-sleep state during LP-WUS monitoring

  - Results in [8A-5] shows that power saving gain -150% when MR is allowed to enter in light-sleep state during LP-WUS monitoring

  - Results in [8A-5] shows that power saving gain -425% when MR is allowed to enter in micro-sleep state during LP-WUS monitoring

- For IM traffic, when WUR ON power setting is equal to 10 units,

- LP-WUS compared with CDRX+ DCP provide

  - Results in [8A-7] shows that power saving gain 56% when MR is allowed to enter in deep-sleep state during LP-WUS monitoring

- For FTP3 traffic,

- LP-WUS compared with CDRX or CDRX+DCP provide

  - Results in [8A-11] [8A-7] [8A-13] show that UPT gain -26%~194% (average: 10.75%) when MR is allowed to enter in deep-sleep state during LP-WUS monitoring

  - Results in [8A-11] [8A-13] show that UPT gain 36%~87% (average: 62.7%) when MR is allowed to enter in light-sleep state during LP-WUS monitoring

  - Results in [8A-11] shows that UPT gain 124%~180% (average: 152%) when MR is allowed to enter in micro-sleep state during LP-WUS monitoring

- LP-WUS compared with R17 PDCCH + CDRX+ DCP provide

  - Results in [8A-5] [8A-13] show that UPT gain -9%~1% (average: -4%) when MR is allowed to enter in deep-sleep state during LP-WUS monitoring

  - Results in [8A-5] [8A-13] show that UPT gain 59%~86% (average: 72.5%) when MR is allowed to enter in light-sleep state during LP-WUS monitoring

  - Results in [8A-5] shows that UPT gain 128% when MR is allowed to enter in micro-sleep state during LP-WUS monitoring

- LP-WUS compared with always-on provides

  - Results in [8A-11] shows that the same UPT performance as always-on when MR is allowed to enter in micro-sleep state during LP-WUS monitoring

- For IM traffic,

- LP-WUS compared with CDRX+ DCP provide

  - Results in [8A-7] show that UPT gain 44%~774% (average: 184%) when MR is allowed to enter in deep-sleep state during LP-WUS monitoring

- LP-WUS compared with R17 PDCCH + CDRX+ DCP provide

  - Results in [8A-5] show that UPT gain 94% when MR is allowed to enter in deep-sleep state during LP-WUS monitoring

  - Results in [8A-5] show that UPT gain 453% when MR is allowed to enter in light-sleep state during LP-WUS monitoring

  - Results in [8A-5] show that UPT gain 2260% when MR is allowed to enter in micro-sleep state during LP-WUS monitoring

For the usage of LP-WUS to replace DCP (i.e., LP-WUS scheme2) for FTP3 traffic,

- The scheme of LP-WUS for DCP replacement compared with CDRX+DCP provide (LP-WUR ON power no more than 1 unit)

  - Results in [8A-2] [8A-8] [8A-11] [8A-15] show that power saving gain (1%~88%, average: 16.75%) when CDRX cycle length is 160ms;

  - Results in [8A-11] [8A-15] show that power saving gain (18%~47%, average: 27%) when CDRX cycle length is 40ms;

  - Results in [8A-2] [8A-8] show that power saving gain (9%~78%, average: 25%) when CDRX cycle length is 320ms;

  - Results in [8A-2] show that power saving gain (10%~63%, average: 34%) when CDRX cycle length is 640ms;

- The scheme of LP-WUS to replace DCP outside DRX active time and to trigger the resumption of MR PDCCH monitoring during DRX active time (including drx_onDurationTimer and drx-inactivityTimer) compared with R17 PDCCH+CDRX+DCP provide

  - Results in [8A-1] show that power saving gain (8%, 29%, 50%) for the case MR enters (micro sleep, light sleep, deep sleep) during LP-WUS monitoring, respectively;

- The scheme of LP-WUS for DCP replacement compared with CDRX+DCP provide

  - Results in [8A-2] [8A-11] [8A-15] show that UPT gain (-6%~0%, average: -1.7%) when CDRX cycle length is 160ms;

  - Results in [8A-11] [8A-15] show that UPT gain (-22%~0%, average: -5.5%) when CDRX cycle length is 40ms;

  - Results in [8A-2] show that the same UPT performance when CDRX cycle length is 320ms;

  - Results in [8A-2] show that the same UPT performance when CDRX cycle length is 640ms;

- The scheme of LP-WUS to replace DCP outside DRX active time and to trigger the resumption of MR PDCCH monitoring during DRX active time (including drx_onDurationTimer and drx-inactivityTimer) compared with R17 PDCCH+CDRX+DCP provide

  - Results in [8A-1] show that UPT gain (34%, -15%, -35%) for the case MR enters (micro sleep, light sleep, deep sleep) during LP-WUS monitoring, respectively;

## 8.2	Coverage

### 8.2.1	Comparison between LP-WUS and NR reference channel

#### 8.2.1.1	Summary of the performance gap between LP-WUS and NR reference channel

The results are summarized in section 8.2.2 and section 8.2.3. A subset of the results are selected to draw observations for comparison between coverage of LP-WUS and reference NR channel. The selection and comparison are performed based on following criterion.

- The relative difference for MIL gap between LP-WUS and NR channel is calculated per each company.

  - If companies only report MIL of NR reference channel or only MIL of LP-WUS, no performance gap can be derived from input from this company.

  - If company only input Msg3 results, and PDCCH results is not provided, only performance gap between LP-WUS and MSG3 can be derived from this company.

- To find whether LP-WUS can achieve similar to or better then MIL of reference NR channels per each companies’ results and then observe the amount of resources required per bit in LP-WUS required for LP-WUS, the following is considered,

- MIL margin of LP-WUS is defined as Y= (MIL of LP-WUS – MIL of NR channel), and when Y>= -1dB, the LP-WUS sample is considered to have similar to or better than reference NR channels.

- If there are multiple samples fulfil above condition from one company, only the sample with minimum resource per bit is kept to draw conclusion.

- The resource required per bit is provided in terms of X MHz*Symbol/bit. For example, if X= 8.64 and WUS BW is 4.32MHz, it means 2 OFDM symbols are required for LP-WUS to achieve MIL similar to or better than that of NR channel.

- If no samples can fulfil the above condition (i.e. Y<-1dB), observation is drawn based on the range of X and Y value across companies.

  - If multiple results were provided by one company, the result with minimum abs(Y) value is selected

The summary of the observation is provided in the following Table 8.2 -1. In each case, only the samples fulfil above condition are collected, where company A: [X (MHz*Symbol/bit), Y (dB)] means company A results show LP-WUS configuration with X MHz*Symbol/bit can achieve MIL similar to or better than MIL of reference channel, and the MIL of LP-WUS – MIL of ref channel is Y dB.

Table 8.2.1 -1 Summary of the performance gap between LP-WUS and NR reference channel

| Company: [X (MHz*Symbol/bit), Y (dB)] |  |  | OOK |  | FSK |  | OFDM |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Results | Observation | Results | Observation | Results | Observation |
| Urban | Normal UE | AL16,4Rx | - | 8.2.2.1 | - | 8.2.2.5 | - | 8.2.2.9 |
|  |  | AL8,4Rx | [8A-13]: [46.08, -0.4] | 8.2.2.1 - X = 46.08, Y = -0.4, reported by 1 source. | - |  | [8A-13]: [34.56, 2.59] [8A-8]: [0.31, 2.08] | 8.2.2.9 - X = 34.56, Y = 2.59, reported by 1 source. - X = 0.31, Y = 2.08, reported by 1 source. |
|  |  | AL16,2Rx | - |  | - |  | [8A-3]: [2.16, -0.71] [8A-8]: [0.31, 2.18] | 8.2.2.9 - X = 2.16, Y = -0.71, reported by 1 source. - X = 0.31, Y = 2.18, reported by 1 source. |
|  |  | AL8,2Rx | [8A-3]: [604.8, 0.91] [8A-11]: [28.8, -0.53] | 8.2.2.1 - X = 28.8, Y = -0.53, reported by 1 source. - X = 604.8, Y = 0.91, reported by 1 source. | - | 8.2.2.5 | [8A-3]:[2.16, 1.79] [8A-8]: [0.31, 5.38] [8A-11]: [18, 0.02] | 8.2.2.9 - X = 0.31~2.16, Y = 1.79~5.38, reported by 1 source. - X = 18, Y = 0.02, reported by 1 source. |
|  |  | Msg3 | [8A-3]: [241.92, -0.96] (Msg3 with 4dB HARQ gain) [8A-5]: [8.64, -0.93] [8A-7]: [17.28, 0.06] [8A-8]: [3.24, 3.23] [8A-11]: [1.8, 2.2] [8A-13]: [2.88, 0.78] [8A-2]: [8.64, 1.79] [8A-6]: [8.64, 11.97] [8A-11](2): [4.32, -0.99] (assumes BS array gain correction for LP-WUS 4dB more than Msg3) [8A-11]: [0.9, 0.68] (ρ=3dB) [8A-14]: [3.24, 1.24] (ρ=3dB) [8A-2]: [4.32, -0.61] (ρ=3dB) | 8.2.2.3 - X = 1.8~8.64, Y = -0.93~11.97, reported by 6 source, no power boosting for LP-WUS; - X = 17.28, Y = 0.06, reported by 1 sources, assumes antenna element gain of LP-WUR is 3dB worse than MR. - X = 241.92, Y = -0.96, reported by 1 sources, assuming 4dB HARQ gain from 2 retransmissions for Msg3 PUSCH and no power boost for LP-WUS. - X=0.9~4.32, Y=-0.61~1.24, reported by 3 sources, assuming 3dB power boosting for LP-WUS. | [8A-2]: [4.32, -0.31] [8A-5]: [4.32, -0.42] [8A-7]: [25.92, -0.34] | 8.2.2.7 - X=4.32, Y=-0.31, reported by 1 source. - X= 4.32, Y=-0.42 reported by 1 source. - X= 25.92, Y=-0.34, reported by one source, assumes antenna element gain of LP-WUR is 3dB worse than MR. | [8A-3]: [2.16, 2.82] (Msg3 with 4dB HARQ gain) [8A-5]: [1.8, 0.06] [8A-7]: [1.08,-0.24] [8A-8]: [0.31, 18.03] [8A-11]: [3.6, 4.84] [8A-13]: [4.32, 5.82] [8A-11](2): [3.6, 0.84] (assumes BS array gain correction for LP-WUS 4dB more than Msg3) | 8.2.2.11 - X= 0.31~4.32, Y=0.06~16.01 reported by 4 sources. - X= 1.08, Y=-0.24, reported by one source, assumes antenna element gain of LP-WUR is 3dB worse than MR. - X=2.16, Y=2.82, reported by 1 source, assuming 4dB HARQ gain from 2 retransmissions for Msg3 PUSCH. |
|  | Redcap UE | AL16,2Rx | - |  | - |  | - |  |
|  |  | AL8, 2Rx | - |  | - |  | - |  |
|  |  | AL16,1Rx | [8A-3]:[302.4, -0.79] [8A-16]: [23.04, 0.47] [8A-2]: [4.32, -0.38] | 8.2.2.2 - X=4.32, Y=-0.31, reported by 1 source. - X = 23.04, Y = 0.47, reported by 1 source. - X = 302.4, Y = -0.79, reported by 1 source. | [8A-2]: [4.32, 2.92] | 8.2.2.6 - X=4.32, Y=2.92, reported by 1 source. | [8A-3]: [2.16,2.92] [8A-11]: [7.2, -0.48] [8A-8]:[0.43, 1.2] | 8.2.2.10 - X=0.43~2.16, Y=1.2~2.92, reported by 2 sources - X=7.2, Y=-0.48, reported by 1 source. |
|  |  | AL8, 1Rx | [8A-3]:[ 120.96, -0.89] [8A-7]: [60.48, -0.92] [8A-8]:[12.96, -0.02] [8A-11]: [7.2, -0.7] [8A-13]:[ 5.76, 0.12] [8A-2]: [4.32, 1.32] | 8.2.2.2 - X=4.32~12.96, Y=-0.02~1.32, reported by 4 sources. - X=60.48~120.96, Y=-0.89~-0.92, reported by 2 sources. | [8A-2]: [4.32, -0.12] [8A-7]:[51.84, 0.18] | - 8.2.2.6X=4.32, Y=-0.12, reported by 1 source. - X= 51.84, Y=0.18, reported by 1 source. | [8A-3]: [2.16, 4.99] [8A-7]: [2.16, 0.48] [8A-8]: [0.31, 9.28] [8A-11]: [3.6, -0.33] [8A-13]: [4.32, 0.97] | 8.2.2.10 - X= 0.31~4.32, Y=-0.33~9.28, reported by 5 sources. |
|  |  | Msg3 | [8A-3]: [241.92, -0.96] (Msg3 with 4dB HARQ gain) [8A-7]: [17.28, 0.06] [8A-16]: [2.88, 0.16] [8A-8]: [3.24, 1.72] [8A-11]: [1.8, 2.2] [8A-13]: [2.88, 0.78] [8A-2]: [8.64, 1.79] [8A-5]: [8.64, -0.93] [8A-11](2):[2.52, 2.24] (assumes BS array gain correction for LP-WUS 4dB more than Msg3) [8A-2]: [4.32, -0.61] (ρ=3dB) [8A-11]: [0.9, 0.68] (ρ=3dB) [8A-14]: [3.24, 1.24] (ρ=3dB) [8A-11](2):[1.68, 2.24] (ρ=3dB) (assumes BS array gain correction for LP-WUS 4dB more than Msg3) | 8.2.2.4 - X=1.8~8.64, Y=-0.93~2.2, reported by 6 sources, no power boosting for LP-WUS. - X=17.28, Y=0.06, reported by 1 sources, no power boosting for LP-WUS, assumes antenna element gain of LP-WUR is 3dB worse than MR. - X=241.92, Y=-0.96, reported by 1 source, assuming 4dB HARQ gain from 2 retransmissions for Msg3 PUSCH. - X=0.9~4.32, Y=-0.61~1.24, reported by 3 sources, assuming 3dB power boosting for LP-WUS. | [8A-2]: [4.32, -0.31] [8A-7]: [25.92, 0.04] [8A-5]: [4.32, -0.43] | 8.2.2.8 - X=4.32, Y=-0.43~-0.31, reported by 2 sources. - X=25.92, Y=0.04, reported by 1 source, assumes antenna element gain of LP-WUR is 3dB worse than MR. | [8A-3]: [2.16, 2.82] (Msg3 with 4dB HARQ gain) [8A-7]: [1.08, -0.24] [8A-8]: [0.31, 18.03] [8A-11]: [3.6, 6.84] [8A-13]: [4.32, 5.82] [8A-5]: [1.8, 0.06] [8A-11](2): [3.6, 2.84] (assumes BS array gain correction for LP-WUS 4dB more than Msg3) | 8.2.2.12 - X= 0.31~4.32, Y=~18.03, reported by 4 sources. - X= 1.08, Y=-0.24, reported by 1 source, assumes antenna element gain of LP-WUR is 3dB worse than MR. - X=2.16, Y=2.82, reported by 1 source, assuming 4dB HARQ gain from 2 retransmissions for Msg3 PUSCH. |
| Rural | Normal UE | AL16,4Rx | - |  | - |  | - |  |
|  |  | AL8,4Rx | - |  | - |  | - |  |
|  |  | AL16,2Rx | [8A-11]: [69.12, -0.96] | 8.2.3.1 - X =69.12, Y =-0.96, reported by 1 source. | - |  | - | 8.2.3.9 |
|  |  | AL8,2Rx | [8A-11]: [28.8, 0.09] [8A-13]: [46.08, -0.25] | 8.2.3.1 - X =28.8 ~ 46.08, Y =-0.25 ~ 0.09, reported by 2 sources. | - |  | [8A-11]: [7.2, -0.29] [8A-13]: [34.56, -0.18] | 8.2.3.9 - X =34.56, Y =-0.18, reported by 1 source. - X =7.2, Y =-0.29, reported by 1 source. |
|  |  | Msg3 | [8A-11]: [1.8, 1.07] [8A-13]: [2.88, 4.71] [8A-6]: [4.32, 11.57] [8A-5]: [8.64, -0.37] [8A-3]: [120.96, 1.56] (Msg3 with 4dB HARQ gain) [8A-11]: [0.9, -0.49] (ρ=3dB) [8A-14]: [3.02, -0.04] (ρ=3dB) | 8.2.3.3 - X =1.8 ~ 8.64, Y =-0.37 ~ 11.57, reported by 4 sources, no power boosting for LP-WUS. - X =0.9 ~ 3.02, Y=-0.49~-0.04, reported by 2 sources, 3dB power boosting for LP-WUS. - X= 120.96, Y= 1.56, reported by 1 source, assuming 4dB HARQ gain from 2 retransmissions for Msg3 PUSCH. | [8A-5]: [4.32, 0.13] | 8.2.3.7 - X =4.32, Y=0.13, reported by 1 source | [8A-11]: [3.6, 3.8] [8A-13]: [2.16, 4.36] [8A-5]: [1.8, 0.62] | 8.2.3.11 - X =1.8 ~ 3.6, Y =0.62 to 4.36, reported by 3 sources. |
|  | Redcap UE | AL16,2Rx | - |  | - |  | - |  |
|  |  | AL8, 2Rx | - |  | - |  | - |  |
|  |  | AL16,1Rx | [8A-16]: [46.08, 1.92] | 8.2.3.2 - X =46.08, Y =1.92, reported by 1 source | - |  | [8A-11]: [7.2, -0.28] | 8.2.3.10 - X =7.2, Y =-0.28, reported by 1 source. |
|  |  | AL8, 1Rx | [8A-11]: [3.78, -0.32] [8A-13]: [11.52, -0.62] | - 8.2.3.2X =3.78 ~ 11.52, Y =-0.62 ~ -0.32, reported by 2 source. | - |  | [8A-11]: [3.6, -0.21] [8A-13]: [17.28, 3.7] | 8.2.3.10 - X =3.6 ~ 17.28, Y =-0.21 ~ 3.7, reported by 2 source. |
|  |  | Msg3 | [8A-16]: [0.18, 0.65] [8A-11]: [1.8, 1.07] [8A-13]: [2.88, 4.71] [8A-5]: [8.64, -0.37] [8A-3]: [120.96, 1.56] (Msg3 with 4dB HARQ gain) [8A-11]:[0.9, -0.49] (ρ=3dB) [8A-14]: [3.02, -0.04] (ρ=3dB) | 8.2.3.4 - X =0.18 ~ 8.64, Y =-0.37 ~ 4.71, reported by 4 sources, no power boosting for LP-WUS. - X=0.9~3.02, Y= -0.49~-0.04, 3dB power boosting for LP-WUS reported by 2 sources. - X= 120.96, Y= 1.56, reported by 1 source, assuming 4dB HARQ gain from 2 retransmissions for Msg3 PUSCH. | [8A-5]: [4.32, 0.13] | 8.2.3.8 - X =4.32, Y=0.13, reported by 1 source | [8A-11]: [3.6, 3.8] [8A-13]: [2.16, 4.36] [8A-5]: [1.8, 0.62] | 8.2.3.12 - X =1.8 ~ 3.6, Y =0.62 to 4.36, reported by 3 sources. |

#### 8.2.1.2	NR Coverage for comparison

The NR coverage for comparison are provided in the following tables

Table 8.2.1 - 2 NR Coverage, Urban, Normal UE

|  | Source | Scenarios | AL16, 4Rx | AL8, 4Rx | AL16, 2Rx | AL8, 2Rx | AL16, 1RX | AL8, 1RX | PUSCH eMBB | PUSCH Msg3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal UE | [8A-11] | Urban 2.6GHz |  |  | 154.94 | 152.33 |  |  | 138.28 | [8A-11]:141.85 [8A-11](2):145.85 |
|  | [8A-15] | Urban 2.6GHz | 166.27 | 164.28 | 162.86 | 160.74 |  |  | 137.62 | 147.20 |
|  | [8A-14] | Urban 4GHz |  |  | 162.65 | 159.84 |  |  | 130.69 | 146.77 |
|  | [8A-5] | Urban 2.6GHz |  |  |  |  |  |  |  | 149.61 |
|  | [8A-9] | Urban 2.6GHz |  |  |  | 163.54 |  |  | 135.88 |  |
|  | [8A-2] | Urban 2.6GHz | 158.80 |  |  |  |  |  |  | 147.83 |
|  | [8A-3] | Urban 2.6GHz | 159.96 | 157.26 | 156.76 | 154.26 |  |  |  | 153.23 |
|  | [8A-8] | Urban 4GHz | 168.30 | 162.21 | 162.11 | 158.91 |  |  |  | 146.26 |
|  | [8A-6] | Urban 2.6Ghz |  |  | 165.55 |  |  |  |  | 135.46 |
|  | [8A-13] | Urban 2.6GHz |  | 162.54 |  |  |  |  |  | 151.53 |
|  | [8A-7] | Urban 2.6GHz | 160.81 | 158.31 |  |  |  |  | 142.51 | 148.73 |

Note: “[8A-11](2)” assumes UL 4dB correction factor better than DL, “[8A-11]” assumes UL and DL same correction factor in coverage evaluation template. For the figures in section 8.2.2 and 8.2.3, the meanings of '[8A-11]' and '[8A-11] 2' refer to this.

Table 8.2.1 - 3 NR Coverage, Urban, Redcap UE

|  | Company Name | Scenarios | AL16, 4Rx | AL8, 4Rx | AL16, 2Rx | AL8, 2Rx | AL16, 1RX | AL8, 1RX | PUSCH eMBB | PUSCH Msg3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Redcap UE | [8A-11] | Urban 2.6GHz |  |  |  |  | 148.72 | 146.02 | 134.96 | [8A-11]:138.85 [8A-11](2): 142.85 |
|  | [8A-15] | Urban 2.6GHz |  |  | 159.86 | 157.74 | 156.61 | 153.17 | 134.62 | 144.20 |
|  | [8A-16] | Urban 2.6GHz |  |  |  |  | 152.96 |  |  | 147.27 |
|  | [8A-17] | Urban 2.6GHz |  |  |  |  | 157.81 |  |  |  |
|  | [8A-2] | Urban 2.6GHz |  |  |  |  | 144.60 | 142.90 |  | 147.83 |
|  | [8A-3] | Urban 2.6GHz |  |  |  |  | 153.76 | 151.06 |  | 153.23 |
|  | [8A-14] | Urban 4GHz |  |  |  |  | 156.40 | 153.11 | 127.69 | 143.77 |
|  | [8A-13] | Urban 2.6GHz |  |  |  |  |  | 153.38 |  | 148.53 |
|  | [8A-7] | Urban 2.6GHz |  |  |  |  | 154.61 | 152.11 | 142.51 | 148.73 |
|  | [8A-8] | Urban 4GHz |  |  |  |  | 155.35 | 152.01 |  | 143.26 |
|  | [8A-5] | Urban 2.6GHz |  |  |  |  |  |  |  | 146.61 |

Note: “[8A-11](2)” assumes UL 4dB correction factor better than DL, “[8A-11]” assumes UL and DL same correction factor in coverage evaluation template. For the figures in section 8.2.2 and 8.2.3, the meanings of '[8A-11]' and '[8A-11] 2' refer to this.

Table 8.2.1 - 4 NR Coverage, Rural, Normal UE

|  | Company Name | Scenarios | AL16, 4Rx | AL8, 4Rx | AL16, 2Rx | AL8, 2Rx | AL16, 1RX | AL8, 1RX | PUSCH eMBB | PUSCH Msg3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal UE | [8A-11] | Rural 700MHz |  |  | 154.99 | 152.32 |  |  | 143.97 | 143.60 |
|  | [8A-15] | Rural 700MHz |  |  | 154.73 | 152.75 |  |  | 138.97 | 140.20 |
|  | [8A-14] | Rural 700MHz |  |  | 157.36 | 154.24 |  |  | 134.78 | 140.76 |
|  | [8A-6] | Rural 700Mhz |  |  |  |  | 154.35 |  |  | 133.27 |
|  | [8A-13] | Rural 700MHz |  |  |  | 150.00 |  |  |  | 135.23 |
|  | [8A-5] | Rural 700MHz |  |  |  |  |  |  |  | 145.3 |
|  | [8A-3] | Rural 700MHz |  |  | 156.63 |  |  |  |  | 148.97 |

Table 8.2.1 - 5 NR Coverage, Rural, Redcap UE

|  | Company Name | Scenarios | AL16, 4Rx | AL8, 4Rx | AL16, 2Rx | AL8, 2Rx | AL16, 1RX | AL8, 1RX | PUSCH eMBB | PUSCH Msg3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Redcap UE | [8A-11] | Rural 700MHz |  |  |  |  | 149.31 | 146.61 | 140.97 | 140.60 |
|  | [8A-15] | Rural 700MHz |  |  | 151.73 | 149.75 | 149.45 | 145.48 | 135.97 | 136.09 |
|  | [8A-16] | Rural 700MHz |  |  |  |  | 153.72 |  |  | 138.99 |
|  | [8A-14] | Rural 700MHz |  |  |  |  | 150.59 | 146.80 | 131.78 | 137.76 |
|  | [8A-13] | Rural 700MHz |  |  |  |  |  | 143.46 |  | 132.23 |
|  | [8A-5] | Rural 700MHz |  |  |  |  |  |  |  | 142.3 |
|  | [8A-3] | Rural 700MHz |  |  |  |  | 152.83 |  |  | 148.97 |

It can be observed that for a given NR channel, there is a MIL difference among companies. One of the reasons contributes to this difference could be the different assumptions for antenna gain corrections for gNB Tx and Rx. Another reason could be different HARQ assumption for MSG3. Another reason could be different link performance between different companies. The assumed antenna gain correction values used by companies for link budget evaluation is summarized as in Table 8.2.1-6

Table 8.2.1-6 Antenna gain correction value (dB) in the link budget evaluation

|  | Urban |  |  |  | Rural |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Company | PDCCH | WUS | Msg3 | PUSCH | PDCCH | WUS | Msg3 | PUSCH |
| [8A-3] | 6 | 6 | 4 | NA | 4 | 4 | 4 | NA |
| [8A-11] | 2.65+8 | 2.65+8 | [8A-11]: 2.65+8 [8A-11](2): 2.65+4 | 0 | 2.65 | 2.65 | 2.65 | 0 |
| [8A-5] | NA | 10.06 | 4.04 | NA | NA | 10.06 | 4.04 | NA |
| [8A-15] | 5.77 | NA | 5.77 | 5.77 | 5.68 | NA | 5.68 | 5.68 |
| [8A-9] | 1.5 | 1.5 | NA | 0 | NA | NA | NA | NA |
| [8A-14] | 5.77 | 3 | 5.77 | 5.77 | 3 | 3 | 0 | NA |
| [8A-16] | 8.05 | 8 | 3 | NA | 0 | 0 | 0 | NA |
| [8A-7] | 8 | 8 | 4.04 | NA | NA | NA | NA | NA |
| [8A-2] | 8 | 8 | 8 | NA | NA | NA | NA | NA |
| [8A-13] | 2.95 | 2.95 | 2.95 | NA | 7.55 | 7.55 | 7.55 | NA |
| [8A-6] | 5.77 | 2.65+8 | 0 | NA | 5.68 | 2.65 | 0 | NA |
| [8A-8] | 5.77 | 5.77 | 5.77 | NA | NA | NA | NA | NA |

### 8.2.2	Results for Urban

In the evaluation, FAR of <=0.1% and <=1% was be used by companies.

The LP-WUR noise figure assumed for coverage evaluation is

- For OOK based LP-WUS, 12~20dB

- For FSK based LP-WUS, 12~20dB

- For OFDM based LP-WUS, 9~15dB

#### 8.2.2.1	OOK, Urban, normal UE, PDCCH

Table 8.2.2.1 - 1 Gap with PDCCH AL16, 4RX, Urban, MR is normal UE

Observation:

For normal UE and urban scenario, comparing MIL of OOK based LP-WUS to legacy PDCCH AL16, 4Rx for paging

- [8A-2], [8A-3], [8A-6], [8A-7] and [8A-8] show that with the LP-WUS resource of X=8.64~604.8 MHz*Symbol/bit , the MIL margin Y is -17.72~-4.79 dB.

Table 8.2.2.1 - 2 Gap with PDCCH AL8, 4RX, Urban, MR is normal UE

Observation:

For normal UE and Urban scenario, comparing MIL of OOK based LP-WUS to legacy PDCCH AL8, 4Rx for paging

- [8A-13] shows it is feasible to achieve comparable MIL (i.e.MIL margin Y>=-1dB) with LP-WUS of X = 46.08 MHz*Symbol/bit, MIL margin Y = -0.4dB

- [8A-3], [8A-7] and [8A-8] show that with the LP-WUS resource of X=12.96~604.8 MHz*Symbol/bit , the MIL margin Y is -7.22~-2.09 dB.

Table 8.2.2.1 - 3 Gap with PDCCH AL8, 2RX, Urban, MR is normal UE

Observation:

For normal UE and Urban scenario, comparing MIL of OOK based LP-WUS to legacy PDCCH AL8, 2Rx for paging

- [8A-3] and [8A-11] show it is feasible to achieve comparable MIL (i.e.MIL margin Y>=-1dB)

  - [8A-11] shows the resource of LP-WUS X = 28.8 MHz*Symbol/bit, MIL margin Y=-0.53dB

  - [8A-3] shows the resource of LP-WUS X = 604.8 MHz*Symbol/bit, MIL margin Y=0.91dB

- [8A-8], [8A-9] and [8A-14] show that with the LP-WUS resource of X=0.72~12.96 MHz*Symbol/bit , the MIL margin Y is -9.76~-3.92 dB.

#### 8.2.2.2	OOK, Urban, Redcap UE, PDCCH

Table 8.2.2.2 - 1 Gap with PDCCH AL16, 1RX

Observation:

For RedCap UE and Urban scenario, comparing MIL of OOK based LP-WUS to legacy PDCCH AL16, 1Rx for paging

- [8A-2], [8A-3] and [8A-16] show it is feasible to achieve comparable MIL (i.e.MIL margin Y>=-1dB)

  - [8A-2] shows the resource of LP-WUS X = 4.32 MHz*Symbol/bit, MIL margin Y=-0.38dB

  - [8A-16] show the resource of LP-WUS X = 23.04 MHz*Symbol/bit, MIL margin Y=0.47dB

  - [8A-3] show the resource of LP-WUS X = 302.4 MHz*Symbol/bit, MIL margin Y=-0.79dB

- [8A-7], [8A-8], [8A-11], [8A-14] and [8A-17] show that with the LP-WUS resource of X=3.02~60.48 MHz*Symbol/bit , the MIL margin Y is -13.85~-1.49 dB.

Table 8.2.2.2 - 2 Gap with PDCCH AL8, 1RX

Observation:

For RedCap UE and Urban scenario, comparing MIL of OOK based LP-WUS to legacy PDCCH AL8, 1Rx for paging

- [8A-2], [8A-3], [8A-7], [8A-8], [8A-11] and [8A-13] show it is feasible to achieve comparable MIL (i.e.MIL margin Y>=-1dB)

  - [8A-2], [8A-11], [8A-13] and [8A-8] show the resource of LP-WUS X = 4.32~12.96 MHz*Symbol/bit, MIL margin Y=-0.02~1.32 dB

  - [8A-3] and [8A-7] show the resource of LP-WUS X = 60.48~120.96 MHz*Symbol/bit, MIL margin Y=-0.89~-0.92dB

- [8A-14] show that with the LP-WUS resource of X=3.02 MHz*Symbol/bit , the MIL margin Y is -3.74 dB.

#### 8.2.2.3	OOK, Urban, Normal UE, Msg.3

Table 8.2.2.3 OOK, Urban, Normal UE, Msg.3

Observation:

For Normal UE in Urban, [8A-2], [8A-3], [8A-5], [8A-6], [8A-7], [8A-8], [8A-11], [8A-13] and [8A-14], show OOK based LP-WUS is feasible to achieve comparable MIL (i.e.MIL margin Y>=-1dB) than PUSCH for Msg.3

- [8A-2], [8A-5], [8A-6], [8A-8], [8A-11] and [8A-13] show that, with LP-WUS resource X = 1.8~8.64 MHz*Symbol/bit, MIL margin Y = -0.93~11.97dB, assuming no power boosting for LP-WUS;

- [8A-7] show that, with LP-WUS resource X=17.28 MHz*Symbol/bit, MIL margin Y=0.06dB, assuming antenna element gain of LP-WUR is 3dB worse than MR.

- [8A-3] show that, with LP-WUS resource X=241.92 MHz*Symbol/bit, MIL margin Y=-0.96dB, assuming 4dB HARQ gain from 2 retransmissions for Msg3 PUSCH and no power boost for LP-WUS

- [8A-2], [8A-11] and [8A-14], show that, with LP-WUS resource X=0.9~4.32 MHz*Symbol/bit, MIL margin Y=-0.61dB~1.24dB, assuming 3dB power boosting for LP-WUS

#### 8.2.2.4	OOK, Urban, Redcap UE, Msg.3

Table 8.2.2.4 OOK, Urban, Redcap UE, Msg.3

Observation:

For Redcap UE in Urban, [8A-2], [8A-3], [8A-5], [8A-7], [8A-8], [8A-11], [8A-13] , [8A-14] and [8A-16] show OOK based LP-WUS is feasible to achieve comparable MIL (i.e.MIL margin Y>=-1dB) than PUSCH for Msg.3

- [8A-2], [8A-5], [8A-8], [8A-11], [8A-13] and [8A-16] show that, with LP-WUS resource X=1.8~8.64 MHz*Symbol/bit, MIL margin Y =-0.93dB~2.2dB, assuming no power boosting for LP-WUS

- [8A-7] show that with LP-WUS resource X=17.28 MHz*Symbol/bit, MIL margin Y=0.06dB, assuming no power boosting for LP-WUS, assumes antenna element gain of LP-WUR is 3dB worse than MR.

- [8A-3] show that with LP-WUS resource X=241.92 MHz*Symbol/bit, MIL margin Y=-0.96dB, assuming no power boosting for LP-WUS and 4dB HARQ gain from 2 retransmissions for Msg3 PUSCH.

- [8A-2], [8A-11] and [8A-14] show that with LP-WUS resource X=0.9~4.32 MHz*Symbol/bit, MIL margin Y=-0.61dB~1.24dB, assuming 3dB power boosting for LP-WUS.

#### 8.2.2.5	FSK, Urban, Normal UE, PDCCH

Table 8.2.2.5 - 1 Gap with PDCCH AL16, 4RX

Observation:

For normal UE and Urban scenario, comparing MIL of FSK based LP-WUS to legacy PDCCH AL16, 4Rx for paging

- [8A-2], [8A-3] and [8A-7] show that with the LP-WUS resource of X=4.32~51.84 MHz*Symbol/bit , the MIL margin Y is -19.79~-5.52 dB.

Table 8.2.2.5 - 2 Gap with PDCCH AL8, 4RX

Observation:

For normal UE and Urban scenario, comparing MIL of FSK based LP-WUS to legacy PDCCH AL8, 4Rx for paging

- [8A-3] and [8A-7] show that with the LP-WUS resource of X=9.72~51.84 MHz*Symbol/bit , the MIL margin Y is -17.09~-6.02 dB.

Table 8.2.2.5 - 3 Gap with PDCCH AL8, 2RX

Observation:

For normal UE and Urban scenario, comparing MIL of FSK based LP-WUS to legacy PDCCH AL8, 2Rx for paging

- [8A-3] and [8A-9] show that with the LP-WUS resource of X=0.72~9.72 MHz*Symbol/bit , the MIL margin Y is -24.14~-14.09 dB.

#### 8.2.2.6	FSK, Urban, Redcap UE, PDCCH

Table 8.2.2.6 - 1 Gap with PDCCH AL16, 1RX

Observation:

For RedCap UE and Urban scenario, comparing MIL of FSK based LP-WUS to legacy PDCCH AL16, 1Rx for paging

- [8A-2] show it is feasible to achieve comparable MIL (i.e.MIL margin Y>=-1dB) with LP-WUS resource X=4.32 MHz*Symbol/bit, MIL margin Y=2.92dB

- [8A-3] and [8A-7] show that with the LP-WUS resource of X=9.72~51.84 MHz*Symbol/bit , the MIL margin Y is -13.59~-2.32 dB.

Table 8.2.2.6 - 2 Gap with PDCCH AL8, 1RX

Observation:

For RedCap UE and Urban scenario, comparing MIL of FSK based LP-WUS to legacy PDCCH AL8, 1Rx for paging

- [8A-2] and [8A-7] show it is feasible to achieve comparable MIL (i.e.MIL margin Y>=-1dB) when

  - [8A-2] with LP-WUS resource X=4.32 MHz*Symbol/bit, MIL margin Y=-0.12dB

  - [8A-7] with LP-WUS resource X=51.84 MHz*Symbol/bit, MIL margin Y=0.18dB

- [8A-3] show that with the LP-WUS resource of X=9.72 MHz*Symbol/bit , the MIL margin Y is -10.89 dB.

#### 8.2.2.7	FSK, Urban, Normal UE, Msg.3

Table 8.2.2.7 Gap with Msg.3

Observation:

For normal UE and Urban scenario, comparing MIL of FSK based LP-WUS to legacy PUSCH for Msg.3,

- [8A-2], [8A-5] and [8A-7] show it is feasible to achieve comparable MIL (i.e.MIL margin Y>=-1dB)

  - [8A-2] with LP-WUS resource X=4.32 MHz*Symbol/bit, MIL margin Y=-0.31dB

  - [8A-5] with LP-WUS resource X=4.32 MHz*Symbol/bit, MIL margin Y=-0.43dB

  - [8A-7] with LP-WUS resource X=25.92 MHz*Symbol/bit, MIL margin Y=-0.34dB, assuming antenna element gain of LP-WUR is 3dB worse than MR.

- [8A-3] show that with the LP-WUS resource of X=9.72 MHz*Symbol/bit , the MIL margin Y is -13.06 dB.

#### 8.2.2.8	FSK, Urban, Redcap UE, Msg.3

Table 8.2.2.8 FSK, Urban, Redcap UE, Msg.3

Observation:

For RedCap UE and Urban scenario, comparing MIL of FSK based LP-WUS to legacy PUSCH for Msg.3,

- [8A-2], [8A-5] and [8A-7] show it is feasible to achieve comparable MIL (i.e.MIL margin Y>=-1dB)

  - [8A-5] and [8A-2] with LP-WUS resource X=4.32 MHz*Symbol/bit, MIL margin Y=-0.43dB~-0.31dB

  - [8A-7] with LP-WUS resource X=25.92 MHz*Symbol/bit, MIL margin Y=-0.01dB, assuming antenna element gain of LP-WUR is 3dB worse than MR

- [8A-3] show that with the LP-WUS resource of X=9.72 MHz*Symbol/bit , the MIL margin Y is -13.06 dB.

#### 8.2.2.9	OFDM, Urban, Normal UE, PDCCH

Table 8.2.2.9-1 Gap with PDCCH AL16, 4RX

Observation:

For normal UE and urban scenario, comparing MIL of OFDM based LP-WUS to legacy PDCCH AL16, 4Rx for paging

- [8A-3], [8A-7] and [8A-8] show that with the LP-WUS resource of X=0.31~8.64 MHz*Symbol/bit , the MIL margin Y is -7.92~-3.91 dB.

Table 8.2.2.9-2 Gap with PDCCH AL16, 2RX

Observation:

For normal UE and Urban scenario, comparing MIL of OFDM based LP-WUS to legacy PDCCH AL16, 2Rx for paging,

- [8A-3] and [8A-8] show it is feasible to achieve comparable MIL (i.e.MIL margin Y>=-1dB)

  - [8A-3] with LP-WUS resource X=2.16 MHz*Symbol/bit, MIL margin Y=-0.71dB

  - [8A-8] with LP-WUS resource X=0.31 MHz*Symbol/bit, MIL margin Y=2.18dB

- [8A-11] shows that with the LP-WUS resource of X=12.96 MHz*Symbol/bit, the MIL margin Y is  -1.91 dB.

Table 8.2.2.9-3 Gap with PDCCH AL8, 4RX

Observation:

For normal UE and Urban scenario, comparing MIL of OFDM based LP-WUS to legacy PDCCH AL8, 4Rx for paging,

- [8A-8] and [8A-13] show it is feasible to achieve comparable MIL (i.e.MIL margin Y>=-1dB)

  - [8A-13] with LP-WUS resource X=34.56 MHz*Symbol/bit, MIL margin Y=2.59 dB

  - [8A-8] with LP-WUS resource X=0.31 MHz*Symbol/bit, MIL margin Y=2.08 dB

- [8A-3] and [8A-7] show that with the LP-WUS resource of X=2.16~8.64 MHz*Symbol/bit, the MIL margin Y is -5.42~-1.21 dB.

Table 8.2.2.9-4 Gap with PDCCH AL8, 2RX

Observation:

For Normal UE in Urban, [8A-3], [8A-8] and [8A-11], show OFDM based LP-WUS is feasible to achieve comparable MIL (i.e. MIL margin Y>=-1dB) than PDCCH AL8 2Rx for paging,

- [8A-3] and [8A-8] show that with LP-WUS resource X=0.31~2.16 MHz*Symbol/bit, MIL margin Y=1.79~5.38 dB

- [8A-11] shows that with LP-WUS resource X=18 MHz*Symbol/bit, MIL margin Y=0.02dB

#### 8.2.2.10	OFDM, Urban, Redcap UE, PDCCH

Table 8.2.2.10-1 Gap with PDCCH AL16, 1RX

Observation:

For Redcap UE and Urban scenario, comparing MIL of OFDM based LP-WUS to legacy PDCCH AL16, 1Rx for paging

- [8A-3], [8A-11] and [8A-8] show that it is feasible to achieve comparable MIL (i.e. MIL margin >=-1dB)

  - [8A-3] and [8A-8] with LP-WUS resource X=0.43~2.16 MHz*Symbol/bit, MIL margin Y=1.2~2.92dB

  - [8A-11] with LP-WUS resource X=7.2 MHz*Symbol/bit, MIL margin Y=-0.48dB

- [8A-7] shows that with the LP-WUS resource of X=8.64 MHz*Symbol/bit, the MIL margin Y is -1.72 dB.

Table 8.2.2.10-2 Gap with PDCCH AL8, 1RX

Observation:

For Redcap UE and Urban scenario, [8A-3], [8A-7], [8A-8], [8A-11] and [8A-13], show OFDM based LP-WUS is feasible to achieve comparable MIL (i.e. MIL margin >=-1dB) than PDCCH AL8 1Rx for paging,

- [8A-3], [8A-7], [8A-8], [8A-11] and [8A-13] show that with LP-WUS resource X= 0.31~4.32 MHz*Symbol/bit, MIL margin Y=-0.33dB~9.28dB

#### 8.2.2.11	OFDM, Urban, Normal UE, Msg.3

Table 8.2.2.11 Gap with PUSCH Msg.3

Observation:

For Normal UE and Urban scenario, [8A-3], [8A-5], [8A-7], [8A-8], [8A-11] and [8A-13], show OFDM based LP-WUS is feasible to achieve comparable MIL (i.e. MIL margin >=-1dB) than PUSCH for Msg.3

- [8A-5], [8A-8], [8A-11] and [8A-13] show that with LP-WUS resource X= 0.31~4.32 MHz*Symbol/bit, MIL margin Y=0.06dB~18.03dB

- [8A-7] shows X= 1.08 MHz*Symbol/bit, MIL margin Y=-0.24dB, assuming antenna element gain of LP-WUR is 3dB worse than MR.

- [8A-3] shows that with LP-WUS resource X= 2.16 MHz*Symbol/bit, MIL margin Y=2.82dB, assuming 4dB HARQ gain from 2 retransmissions for Msg3 PUSCH

#### 8.2.2.12	OFDM, Urban, Redcap UE, Msg.3

Table 8.2.2.12 OFDM, Urban, Redcap UE, Msg.3

Observation:

For RedCap UE and Urban scenario, [8A-3], [8A-5], [8A-7], [8A-8], [8A-11] and [8A-13] show OFDM based LP-WUS is feasible to achieve comparable MIL (i.e. MIL margin >=-1dB) than PUSCH for Msg.3

- [8A-5], [8A-8], [8A-11] and [8A-13] show that with LP-WUS resource X= 0.31~4.32 MHz*Symbol/bit, MIL margin Y=0.06dB~18.03dB

- [8A-7] shows that with LP-WUS resource X= 1.08 MHz*Symbol/bit, MIL margin Y=-0.24dB, assuming antenna element gain of LP-WUR is 3dB worse than MR

- [8A-3] shows that with LP-WUS resource X= 2.16 MHz*Symbol/bit, MIL margin Y=2.82dB, assuming 4dB HARQ gain from 2 retransmissions for Msg3 PUSCH

### 8.2.3	Results for Rural

In the evaluation, FAR of 0.1% and 1%  are used by companies.

The LP-WUR noise figure assumed for coverage evaluation is

- For OOK based LP-WUS, 12~15dB

- For OFDM based LP-WUS, 9.5~12dB

#### 8.2.3.1	OOK, Rural, normal UE, PDCCH

- Gap with AL16, 2Rx

Observation:

For normal UE and Rural scenario, comparing MIL of OOK based LP-WUS to legacy PDCCH AL16, 2Rx for paging

- [8A-11] shows that it is feasible to achieve comparable MIL (i.e. MIL margin >=-1dB) when LP-WUS resource X=69.12 MHz*Symbol/bit, MIL margin Y=-0.96dB

- [8A-3], [8A-6] and [8A-14] show that with the LP-WUS resource of X = 3.02 ~ 120.96 MHz*Symbol/bit, the MIL margin Y is -16.64~-5.71 dB.

- Gap with AL8, 2Rx

Observation:

For normal UE and Rural scenario, comparing MIL of OOK based LP-WUS to legacy PDCCH AL8, 2Rx for paging

- [8A-11] and [8A-13] show that it is feasible to achieve comparable MIL (i.e. MIL margin >=-1dB) when LP-WUS resource X=28.8~46.08 MHz*Symbol/bit, MIL margin Y=-0.25~0.09dB

- [8A-14] shows that with the LP-WUS resource of X=3.02 MHz*Symbol/bit, the MIL margin Y is -13.52 dB.

#### 8.2.3.2	OOK, Rural, Redcap UE, PDCCH

- Gap with AL16, 1Rx

Observation:

For RedCap UE and Rural scenario, comparing MIL of OOK based LP-WUS to legacy PDCCH AL16, 1Rx for paging

- [8A-16] shows that it is feasible to achieve comparable MIL (i.e. MIL margin >=-1dB) when LP-WUS resource X=46.08 MHz*Symbol/bit, MIL margin Y=1.92dB

- [8A-3], [8A-11] and [8A-14] show that with the LP-WUS resource of X=3.02~120.96 MHz*Symbol/bit, the MIL margin Y is -12.87~-1.08 dB.

- Gap with AL8, 1Rx

Observation:

For RedCap UE and Rural scenario, comparing MIL of OOK based LP-WUS to legacy PDCCH AL8, 1Rx for paging

- [8A-11] and [8A-13] show that it is feasible to achieve comparable MIL (i.e. MIL margin >=-1dB) when LP-WUS resource X=3.78~11.52 MHz*Symbol/bit, MIL margin Y=-0.62dB~-0.32dB

- [8A-14] shows that with the LP-WUS resource of X=3.02 MHz*Symbol/bit, the MIL margin Y is -9.08 dB.

#### 8.2.3.3	OOK, Rural, Normal UE, Msg.3

Observation:

For Normal UE in Rural, based on the results, [8A-3], [8A-5], [8A-6], [8A-11], [8A-13] and [8A-14] show OOK based LP-WUS is feasible to achieve comparable MIL (i.e. MIL margin Y>=-1dB) than MSG3 PUSCH,

- [8A-5], [8A-6], [8A-11] and [8A-13] show that with LP-WUS resource X=1.8~8.64 MHz*Symbol/bit, MIL margin Y=-0.37B~11.57dB

- [8A-11] and [8A-14] show that with LP-WUS resource X=0.9~3.02 MHz*Symbol/bit, MIL margin Y=-0.49dB~-0.04dB, assuming 3dB power boosting for LP-WUS.

- [8A-3] shows that with LP-WUS resource X=120.96 MHz*Symbol/bit, MIL margin Y=1.56dB, assuming 4dB HARQ gain from 2 retransmissions for Msg3 PUSCH.

#### 8.2.3.4	OOK, Rural, Redcap UE, Msg.3

Observation:

For RedCap UE in Rural, based on the results, [8A-3], [8A-5], [8A-16], [8A-11], [8A-13] and [8A-14] show OOK based LP-WUS is feasible to achieve comparable MIL (i.e. MIL margin Y>=-1dB) than MSG3 PUSCH,

- [8A-5], [8A-11], [8A-13] and [8A-16] show that with LP-WUS resource X = 0.18 ~ 8.64 MHz*Symbol/bit, MIL margin Y=-0.37dB~4.71dB, assuming no power boosting for LP-WUS

- [8A-11] and [8A-14] show that with LP-WUS resource X=0.9~3.02 MHz*Symbol/bit, MIL margin Y=-0.49dB~-0.04dB, assuming 3dB power boosting for LP-WUS

- [8A-3] shows that with LP-WUS resource X=120.96 MHz*Symbol/bit, MIL margin Y=1.56dB, assuming 4dB HARQ gain from 2 retransmissions for Msg3 PUSCH.

#### 8.2.3.5	FSK, Rural, normal UE, PDCCH

No input

#### 8.2.3.6	FSK, Rural, Redcap UE, PDCCH

No input

#### 8.2.3.7	FSK, Rural, Normal UE, Msg.3

Observation:

For normal UE and Rural scenario, [8A-5] shows FSK based LP-WUS is feasible to achieve comparable MIL (i.e. MIL margin Y>=-1dB) than MSG3 PUSCH when

- LP-WUS resource X=4.32MHz*Symbol/bit, MIL margin Y=0.13dB

#### 8.2.3.8	FSK, Rural, Redcap UE, Msg.3

Observation:

For RedCap UE and Rural scenario, [8A-5] shows FSK based LP-WUS is feasible to achieve comparable MIL (i.e. MIL margin Y>=-1dB) than MSG3 PUSCH when

- LP-WUS resource X=4.32MHz*Symbol/bit, MIL margin Y=0.13dB

#### 8.2.3.9	OFDM, Rural, normal UE, PDCCH

- Gap with PDCCH AL16, 2Rx

Observation:

For normal UE and Rural scenario, comparing MIL of OFDM based LP-WUS to legacy PDCCH AL16, 2Rx

- [8A-11] show that with the LP-WUS resource of X=17.28 MHz*Symbol/bit, the MIL margin Y is  -2.36 dB.

- Gap with PDCCH AL8 2Rx

Observation:

For normal UE and Rural scenario, [8A-11] and [8A-13] show OFDM based LP-WUS is feasible to achieve comparable MIL (i.e. MIL margin Y>=-1dB) than MSG3 PUSCH

- [8A-13] shows that with LP-WUS resource X=34.56 MHz*Symbol/bit, MIL margin Y=-0.18dB

- [8A-11] shows that with LP-WUS resource X=7.2MHz*Symbol/bit, MIL margin Y=-0.29dB

#### 8.2.3.10	OFDM, Rural, Redcap UE, PDCCH

- Gap with PDCCH AL16 1Rx

Observation:

For Redcap UE in Rural, [8A-11] shows OFDM based LP-WUS is feasible to achieve comparable MIL (i.e. MIL margin Y>=-1dB) than PDCCH AL16 1Rx for paging when

- LP-WUS resource X=7.2 MHz*Symbol/bit, MIL margin Y=-0.28dB

- Gap with PDCCH AL8 1Rx

Observation:

For Redcap UE in Rural, [8A-11] and [8A-13] show OFDM based LP-WUS is feasible to achieve comparable MIL (i.e. MIL margin Y>=-1dB) than PDCCH AL8 1Rx for paging when

- LP-WUS resource X=3.6~17.28 MHz*Symbol/bit, MIL margin Y=-0.21dB~3.7dB

#### 8.2.3.11	OFDM, Rural, Normal UE, Msg.3

Observation:

For normal UE in Rural, [8A-5], [8A-11] and [8A-13] show OFDM based LP-WUS is feasible to achieve comparable MIL (i.e. MIL margin Y>=-1dB) than PUSCH for MSG.3 when

- LP-WUS resource X=1.8~3.6 MHz*Symbol/bit, MIL margin Y=0.62dB~4.36dB

#### 8.2.3.12	OFDM, Rural, Redcap UE, Msg.3

Observation:

For RedCap UE in Rural, [8A-5], [8A-11] and [8A-13] show OFDM based LP-WUS is feasible to achieve comparable MIL (i.e. MIL margin Y>=-1dB) than PUSCH for MSG.3 when

- LP-WUS resource X=1.8~3.6 MHz*Symbol/bit, MIL margin Y=0.62dB~4.36dB

## 8.3	LLS results observations

In this section and sub-sections it is assumed that “tolerate” means that SNR degradation <= 2dB compared to error free case. Section is based on contributed results in [8B-29].

### 8.3.1	Timing error

#### 8.3.1.1	Results for timing error (without sliding window)

The results are generated so that script finds a pair of results from a single source that differ only in timing error column. The SNR degradation is extracted, and the smallest degradation shown by a source is taken as representative value for the source.

For the following Waveforms:

- OOK-1 30kHz SCS

- OOK-1 60kHz SCS

- OOK-1 120kHz SCS

- OOK-2 M=2

- OOK-2 M=4

- OOK-3

- OOK-4 M=2

- OOK-4 M=4

- OOK-4 M>4

- FSK M=1

- FSK-1 M=2

- FSK-2 M=2

- OFDMA

With the following assumption

- ADC bit-width is 4 or more bits.

- Frequency error is 0 ppm.

- Other parameters are not restricted.

Results are captured in Table 8.3-1.

Table 8.3-1 Timing error without sliding window.

| Te [us] | OOK-1 30kHz SCS |  | OOK-1 60kHz SCS |  | OOK-1 120kHz SCS |  | OOK-3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNR deg [dB] | <2dB sources | >=2dB sources | <2dB sources | >=2dB sources | <2dB sources | >=2dB sources | <2dB sources | >=2dB sources |
| 1 | [8B-7][8B-11] [8B-17][8B-2] [8B-24][8B-27] |  | [8B-2][8B-17] [8B-7][8B-28] |  | [8B-17][8B-2] [8B-28] |  |  |  |
| 2 | [8B-7][8B-11] [8B-17][8B-2] [8B-24][8B-27] |  | [8B-7][8B-1][8B-21] [8B-17][8B-2][8B-28] |  | [8B-28] | [8B-1][8B-17] [8B-2][8B-21] | [8B-15] |  |
| 3 | [8B-17] |  | [8B-17] |  |  | [8B-17] | [8B-15] |  |
| 4 | [8B-7][8B-1] [8B-11][8B-17] [8B-2][8B-24][8B-27] |  | [8B-1][8B-17][8B-7] | [8B-21][8B-2] [8B-28] |  | [8B-17][8B-2] [8B-28][8B-21] | [8B-15] |  |
| 5 | [8B-3] |  |  |  |  |  |  |  |
| 10 |  | [8B-3] |  |  |  |  |  |  |
| 15 |  | [8B-3] |  |  |  |  |  |  |
| 20 |  | [8B-3] |  |  |  |  |  |  |
| Sampling rate range [MHz] | 0,96- 15,36 |  | 1,12- 7,68 |  | 1,12- 7,68 |  | 30,72- 30,72 |  |
| Inner GB BW range, if applicable [kHz] | 0- 180 |  | N/A |  | N/A |  | N/A |  |
| Total Source | [8B-7][8B-1][8B-11][8B-17] [8B-2][8B-3][8B-24][8B-27] |  | [8B-7][8B-1][8B-21] [8B-17][8B-2][8B-28] |  | [8B-1][8B-21][8B-17] [8B-2][8B-28] |  | [8B-15] |  |

| Te [us] | OOK-2 M=2 |  | OOK-2 M=4 |  | OOK-4 M=2 |  | OOK-4 M=4 |  | OOK-4 M>4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNR deg [dB] | <2dB sources | >=2dB sources | <2dB sources | >=2dB sources | <2dB sources | >=2dB sources | <2dB sources | >=2dB sources | <2dB sources | >=2dB sources |
| 1 | [8B-8][8B-7] [8B-11][8B-17] [8B-2][8B-24] |  | [8B-8][8B-7] [8B-1][8B-11] [8B-17][8B-2] [8B-24] |  | [8B-7][8B-11] [8B-17][8B-2] [8B-24][8B-27] |  | [8B-7][8B-1] [8B-17][8B-2] [8B-3][8B-24] | [8B-11] | [8B-3] | [8B-1] |
| 2 | [8B-8][8B-7] [8B-11][8B-17] [8B-2][8B-24] |  | [8B-8][8B-1] [8B-11][8B-17] [8B-2][8B-24] | [8B-7] | [8B-7][8B-1] [8B-21][8B-17] [8B-2][8B-24] [8B-27] | [8B-11] | [8B-13][8B-3] [8B-7] | [8B-1][8B-11] [8B-21][8B-17] [8B-2][8B-24] |  | [8B-3] |
| 3 | [8B-17] |  | [8B-17] |  | [8B-17] |  | [8B-3] | [8B-17] |  | [8B-3] |
| 4 | [8B-8][8B-17] [8B-2][8B-24] | [8B-11] [8B-7] | [8B-8][8B-17] [8B-2] | [8B-11][8B-7] [8B-24] | [8B-7][8B-27] | [8B-11][8B-21] [8B-17][8B-2] [8B-24] | [8B-7] | [8B-11][8B-21] [8B-17][8B-2] [8B-3][8B-24] |  | [8B-3] |
| 5 |  |  |  |  | [8B-3] |  |  | [8B-3] |  | [8B-3] |
| 10 |  |  |  |  |  | [8B-3] |  | [8B-3] |  | [8B-3] |
| 15 |  |  |  |  |  | [8B-3] |  | [8B-3] |  | [8B-3] |
| 20 |  |  |  |  |  | [8B-3] |  | [8B-3] |  | [8B-3] |
| Sampling rate range [MHz] | 0,96- 15,36 |  | 0,96- 15,36 |  | 1,92- 15,36 |  | 1,92- 15,36 |  | 3,4- 7,68 |  |
| Inner GB BW range, if applicable [kHz] | 120- 480 |  | 120- 480 |  | N/A |  | N/A |  | N/A |  |
| Total Sources | [8B-7][8B-11] [8B-17][8B-2][8B-24] |  | [8B-7][8B-1][8B-11] [8B-17][8B-2][8B-24] |  | [8B-7][8B-1][8B-11][8B-21] [8B-17][8B-2][8B-3] [8B-24][8B-27] |  | [8B-7][8B-1][8B-11][8B-21] [8B-17][8B-13][8B-2] [8B-3][8B-24] |  | [8B-1][8B-3] |  |

| Te [us] | FSK M=1 |  | FSK-1 M=2 |  | FSK-2 M=2 |  | OFDMA |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNR deg [dB] | <2dB sources | >=2dB sources | <2dB sources | >=2dB sources | <2dB sources | >=2dB sources | <2dB sources | >=2dB sources |
| 1 | [8B-7] [8B-17][8B-24] [8B-27] |  | [8B-8][8B-17] [8B-7][8B-24] |  | [8B-7] [8B-11] [8B-1] [8B-17][8B-2] [8B-24] |  | [8B-28] | 0 |
| 2 | [8B-17] [8B-7][8B-27] | [8B-24] | [8B-17] | [8B-7][8B-24] | [8B-17][8B-2] [8B-11] [8B-1] | [8B-7] [8B-24] | [8B-28] | 0 |
| 3 | [8B-17] |  | [8B-17] |  | [8B-1] [8B-17] |  |  |  |
| 4 | [8B-17][8B-27] | [8B-7][8B-24] | [8B-17] | [8B-7][8B-24] | [8B-1][8B-17] [8B-2] | [8B-11] [8B-7][8B-24] | [8B-28] | 0 |
| 10 | 0 |  | 0 |  | 0 |  |  |  |
| Sampling rate range [MHz] | 0,96- 8,64 |  | 0,96- 8,64 |  | 0,96- 15,36 |  | 3,84- 7,68 |  |
| Inner GB BW range, if applicable [kHz] | 0- 480 |  | 180- 480 |  | 0- 480 |  | N/A |  |
| Total Sources | [8B-7][8B-17] [8B-24][8B-27] |  | [8B-17][8B-7][8B-24] |  | [8B-7][8B-1][8B-11] [8B-17][8B-2][8B-24] |  | [8B-28] |  |

Table 8.3-2 Tolerance to timing error by waveform

| Waveform | Tolerance up to timing error [us] | Note |
| --- | --- | --- |
| OOK-1 30kHz SCS | 5 |  |
| OOK-1 60kHz SCS | 3 |  |
| OOK-1 120kHz SCS | 1 |  |
| OOK-3 | 4 | single source |
| OOK-2 M=2 | 4 |  |
| OOK-2 M=4 | 3 |  |
| OOK-4 M=2 | 3 |  |
| OOK-4 M=4 | 1 | [8B-2] shows that performance degrade more than 2dB for 1us, with assumption of TDL-C 1000ns, but degrades <2dB for 1us with assumption of TDL-C 300ns. The 300ns delay spread was baseline. |
| OOK-4 M>4 | 1 |  |
| FSK M=1 | 2 |  |
| FSK-1 M=2 | 1 |  |
| FSK-2 M=2 | 2/3 | [8B-1] [8B-17] sources showed tolerance up to 3us. |
| OFDMA | 4 | [8B-28] shows that OFDMA can be tolerant up to 4us. |

#### 8.3.1.1A	Observation for timing error (without sliding window):

- There are the following observations for timing error from Table 8.3-2:

  - OOK/FSK waveform with longer time segment and with a single or multiple frequency segments is comparable (0 us) or more robust (up to 4us) to timing error than waveform with shorter time segment.

  - [8B-28] shows that without sliding window, OFDMA tolerates up to 4us timing error, when SCS <=30kHz, and receiver did not perform FFT.

#### 8.3.1.2	Results for timing error (with sliding window)

For the following Waveforms:

- OOK-1 30kHz SCS

- OOK-1 60kHz SCS

- OOK-1 120kHz SCS

- OOK-2 M=2

- OOK-2 M=4

- OOK-3

- OOK-4 M=2

- OOK-4 M=4

- OOK-4 M>4

- FSK M=1

- FSK-1 M=2

- FSK-2 M=2

- OFDMA

With the following assumption

- ADC bit-width is 4 or more bits.

- Frequency error is 0 ppm.

- Other parameters are not restricted.

Results are presented in Table 8.3-3:

Table 8.3-3 Timing error with time sliding window

| Te [us] | OOK-1 30kHz SCS |  | OOK-2 M=2 |  | OOK-2 M=4 |  |
| --- | --- | --- | --- | --- | --- | --- |
| SNR deg [dB] | <2dB sources | >=2dB sources | <2dB sources | >=2dB sources | <2dB sources | >=2dB sources |
| 1 | [8B-8] |  | [8B-8] |  | [8B-8] |  |
| 2 | [8B-8] |  | [8B-8] |  | [8B-8] |  |
| 4 | [8B-8] |  | [8B-8] |  | [8B-8] |  |
| 10 | [8B-8] |  | [8B-8] |  | [8B-8] |  |
| Sampling rate range [MHz] | 0,96- 7,68 |  | 0,96- 7,68 |  | 0,96- 7,68 |  |
| Inner GB BW range, if applicable [kHz] | N/A |  | 180- 180 |  | 180- 180 |  |
| Total Source | [8B-8] |  | [8B-8] |  | [8B-8] |  |

| Te [us] | FSK M=1 |  | FSK-1 M=2 |  | FSK-2 M=2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| SNR deg [dB] | <2dB sources | >=2dB sources | <2dB sources | >=2dB sources | <2dB sources | >=2dB sources |
| 1 | [8B-8] |  | [8B-8] |  | [8B-8] |  |
| 2 | [8B-8] |  |  | [8B-8] |  | [8B-8] |
| 3 |  |  |  |  |  |  |
| 4 |  | [8B-8] |  | [8B-8] |  | [8B-8] |
| 10 |  | [8B-8] |  | [8B-8] |  | [8B-8] |
| Sampling rate range [MHz] | 0,96- 7,68 |  | 0,96- 7,68 |  | 0,96- 7,68 |  |
| Inner GB BW range, if applicable [kHz] | 180- 180 |  | 180- 180 |  | 180- 180 |  |
| Total Sources | [8B-8] |  | [8B-8] |  | [8B-8] |  |

| Te [us] | OFDMA |  |
| --- | --- | --- |
| SNR deg [dB] | <2dB sources | >=2dB sources |
| 1 | [8B-2][8B-21] [8B-15] |  |
| 2 | [8B-25][8B-17][8B-13][8B-15] | [8B-21] |
| 3 |  | [8B-15] |
| 4 | [8B-25][8B-21] |  |
| 10 |  |  |
| Sampling rate range [MHz] | 3,84 30,72 |  |
| Inner GB BW range, if applicable [kHz] | N/A |  |
| Total Sources | [8B-15][8B-21][8B-17][8B-2] [8B-13][8B-25] |  |

#### 8.3.1.2A	Observation for timing error (with sliding window):

There are the following observations for timing error in Table 8.3-3:

- For OFDMA, tolerance to timing error varies with sliding window size assumed by a receiver and was shown to tolerate timing error up to 4us if proper sliding window size is assumed by a receiver.

### 8.3.2	Frequency error

#### 8.3.2.1	Results for frequency error

The results are generated so that script finds a pair of results from a single source that differ only in frequency error column. The SNR degradation is extracted, and the smallest degradation shown by company is taken as representative value for the source.

For the following Waveforms:

- OOK-1 30kHz SCS

- OOK-1 60kHz SCS

- OOK-1 120kHz SCS

- OOK-2 M=2

- OOK-2 M=4

- OOK-3

- OOK-4 M=2

- OOK-4 M=4

- OOK-4 M>4

- FSK M=1

- FSK-1 M=2

- FSK-2 M=2

- OFDMA

With the following assumptions:

- ADC bit-width is 4 or more bits.

- Timing error is 0 micro sec.

- Other parameters are not restricted.

Results are presented in Table 8.3-4:

Table 8.3-4 Frequency error robustness for different waveforms

| Fe [kHz] | OOK-1 30kHz SCS |  | Fe [kHz] | OOK-1 60kHz SCS |  | Fe [kHz] | OOK-1 120kHz SCS |  | Fe [kHz] | OOK-2 M=2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | <2dB sources | >=2dB sources |  | <2dB sources | >=2dB sources |  | <2dB sources | >=2dB sources |  | <2dB sources | >=2dB sources |
| 2 | [8B-17] |  | 2 | [8B-17] |  | 2 | [8B-17] |  | 2 | [8B-17] |  |
| 2,6 | [8B-8] |  | 4 | [8B-17] |  | 4 | [8B-17] |  | 2,6 | [8B-8] |  |
| 4 | [8B-17] |  | 10 | [8B-17] |  | 10 | [8B-17] |  | 4 | [8B-17] |  |
| 5,2 | [8B-8] |  | 13 | [8B-2][8B-7] |  | 20 | [8B-17] |  | 5,2 | [8B-8] |  |
| 10 | [8B-17] |  | 20 | [8B-17] |  | 26 | [8B-2] |  | 13 | [8B-8][8B-2] [8B-7] |  |
| 13 | [8B-8][8B-27] [8B-2] [8B-7] |  | 26 | [8B-2] |  | 40 | [8B-21] |  | 20 | [8B-17] |  |
| 20 | [8B-17] |  | 40 | [8B-21] |  | 100 | [8B-17] |  | 26 | [8B-8][8B-2] |  |
| 26 | [8B-8][8B-27] [8B-2] |  | 100 | [8B-17] |  | 200 | [8B-17] |  | 40 | [8B-24] |  |
| 40 | [8B-24] |  | 200 | [8B-7] |  | 260 | [8B-1] |  | 52 | [8B-7] |  |
| 100 | [8B-17] |  | 260 | [8B-17] |  | 300 | [8B-17] |  | 100 | [8B-17] |  |
| 130 | [8B-22][8B-8] [8B-11][8B-3] [8B-7][8B-27] |  | 300 | [8B-7][8B-1] |  | 400 | [8B-21][8B-17] |  | 130 | [8B-8][8B-25] [8B-7][8B-11] |  |
| 200 | [8B-24][8B-17] |  | 400 | [8B-17] |  |  |  |  | 200 | [8B-24][8B-17] |  |
| 260 | [8B-1][8B-8] [8B-11][8B-3] [8B-7] |  |  |  |  |  |  |  | 260 | [8B-8][8B-11] [8B-7] |  |
| 300 | [8B-17] |  |  |  |  |  |  |  | 300 | [8B-17] |  |
| 390 | [8B-3] |  |  |  |  |  |  |  | 400 |  | [8B-24] |
| 400 | [8B-24][8B-17] |  |  |  |  |  |  |  | 520 |  | [8B-25][8B-11] |
| 520 | [8B-11][8B-3] [8B-22][8B-7] |  |  |  |  |  |  |  | 600 |  | [8B-24] |
| 600 | [8B-24] |  |  |  |  |  |  |  | 800 |  | [8B-24] |
| 800 | [8B-24] |  |  |  |  |  |  |  |  |  |  |
| Sampling rate range [MHz] | 0,96- 15,36 |  |  | 1,12- 4,32 |  |  | 1,12- 4,32 |  |  | 0,96- 15,36 |  |
| Inner GB BW range, if applicable [kHz] | N/A |  |  | N/A |  |  | N/A |  |  | 120- 480 |  |
| # of sources with/wo time domain sliding window | [8B-8]/ [8B-24][8B-17] [8B-22][8B-1] [8B-2][8B-11] [8B-3][8B-7][8B-27] |  |  | [-]/ [8B-17][8B-21] [8B-1][8B-2][8B-7] |  |  | [-] [8B-21][8B-17][8B-2][8B-1] |  |  | [8B-8]/ [8B-24][8B-17][8B-25] [8B-2][8B-11][8B-7] |  |
| # of sources with/wo frequency adjustment | [-]/ [8B-8][8B-24][8B-17] [8B-22][8B-1] [8B-2][8B-11] [8B-3][8B-7][8B-27] |  |  | [-]/ [8B-17][8B-21] [8B-1][8B-2][8B-7] |  |  | [-] [8B-21][8B-17][8B-2][8B-1] |  |  | [-]/ [8B-24][8B-17][8B-25] [8B-8][8B-2][8B-11][8B-7] |  |

| Fe [kHz] | OOK-2 M=4 |  | Fe [kHz] | OOK=4 M=2 |  | Fe [kHz] | OOK=4 M=4 |  | Fe [kHz] | OOK=4 M>4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | <2dB sources | >=2dB sources |  | <2dB sources | >=2dB sources |  | <2dB sources | >=2dB sources |  | <2dB sources | >=2dB sources |
| 2 | [8B-17] |  | 2 | [8B-17] |  | 2 | [8B-17] |  | 130 | [8B-3] |  |
| 2,6 | [8B-8] |  | 4 | [8B-17] |  | 4 | [8B-17] |  | 260 | [8B-3][8B-1] |  |
| 4 | [8B-17] |  | 10 | [8B-17] |  | 7 | [8B-13] |  | 390 | [8B-3] |  |
| 5,2 | [8B-8] |  | 13 | [8B-27][8B-2] [8B-7] |  | 10 | [8B-17] |  |  |  |  |
| 13 | [8B-8][8B-2] [8B-7] |  | 20 | [8B-17] |  | 13 | [8B-2] |  |  |  |  |
| 20 | [8B-17] |  | 26 | [8B-27][8B-2] |  | 20 | [8B-17] |  |  |  |  |
| 26 | [8B-8][8B-2] [8B-1] |  | 32 | [8B-24] |  | 26 | [8B-13][8B-2] |  |  |  |  |
| 40 | [8B-24] |  | 40 | [8B-21][8B-24] |  | 40 | [8B-21][8B-24] [8B-13] |  |  |  |  |
| 52 | [8B-7] |  | 100 | [8B-17] |  | 52 | [8B-2] |  |  |  |  |
| 100 | [8B-17] |  | 130 | [8B-25][8B-3] [8B-11][8B-7] [8B-27] |  | 100 | [8B-17] |  |  |  |  |
| 130 | [8B-8][8B-11] [8B-22] | [8B-7] | 200 | [8B-24][8B-17] |  | 130 | [8B-25][8B-22] [8B-3][8B-2] [8B-11] |  |  |  |  |
| 200 | [8B-24][8B-17] |  | 260 | [8B-11][8B-3] [8B-7][8B-1] |  | 200 | [8B-24][8B-17] |  |  |  |  |
| 260 |  | [8B-8][8B-11][8B-7] | 300 | [8B-24] |  | 260 | [8B-11][8B-3] [8B-1] |  |  |  |  |
| 300 | [8B-17] |  | 390 | [8B-3] |  | 390 | [8B-3] |  |  |  |  |
| 400 | [8B-24] |  | 400 | [8B-21][8B-24] [8B-17] |  | 400 | [8B-21][8B-24][8B-17] |  |  |  |  |
| 520 |  | [8B-11][8B-22] | 520 | [8B-11][8B-3][8B-7] |  | 520 | [8B-11][8B-3] [8B-22] |  |  |  |  |
| 600 | [8B-24] |  | 600 | [8B-24] |  | 600 | [8B-24] |  |  |  |  |
| 800 | [8B-24] |  | 800 | [8B-24] |  | 800 | [8B-24] |  |  |  |  |
|  |  |  | 1200 | [8B-24] |  |  |  |  |  |  |  |
| Sampling rate range [MHz] | 0,96- 15,36 |  |  | 1,92- 15,36 |  |  | 1,92- 15,36 |  |  | 3,4- 7,68 |  |
| Inner GB BW range, if applicable [kHz] | 120- 480 |  |  | N/A |  |  | N/A |  |  | N/A |  |
| # of sources with/wo time domain sliding window | [8B-8]/ [8B-24][8B-17][8B-22] [8B-1][8B-8][8B-2][8B-11][8B-7] |  |  | [-]/ [8B-24][8B-17][8B-25] [8B-21][8B-1][8B-2][8B-11] [8B-3][8B-7][8B-27] |  |  | [-]/ [8B-24][8B-17][8B-25] [8B-13][8B-21][8B-22] [8B-1][8B-2][8B-11][8B-3] |  |  | [-] / [8B-3][8B-1]. |
| # of sources with/wo frequency adjustment | [-]/ [8B-24][8B-17][8B-22] [8B-1][8B-8][8B-2] [8B-11][8B-7] |  |  | [-]/ [8B-24][8B-17][8B-25] [8B-21][8B-1][8B-2] [8B-11][8B-3][8B-7][8B-27] |  |  | [-]/ [8B-24][8B-17][8B-25] [8B-13][8B-21][8B-22] [8B-1][8B-2][8B-11][8B-3] |  |  | [-]/. [8B-3][8B-1]. |

| Fe [kHz] | FSK M=1 |  | Fe [kHz] | FSK-1 M=2 |  | Fe [kHz] | FSK-2 M=2 |  | Fe [kHz] | OFDMA |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | <2dB sources | >=2dB sources |  | <2dB sources | >=2dB sources |  | <2dB sources | >=2dB sources |  | <2dB sources | >=2dB sources |
| 2,6 | [8B-8][8B-17] [8B-27] |  | 2,6 | [8B-8] | 0 | 2,6 | [8B-8][8B-17] |  | 1,4 | [8B-13] | 0 |
| 5,2 | [8B-8][8B-17] [8B-27] |  | 5,2 | [8B-8][8B-17] | 0 | 5,2 | [8B-8][8B-17] |  | 2 | [8B-17] | 0 |
| 13 | [8B-8][8B-27] [8B-7] |  | 13 | [8B-8][8B-7] | 0 | 13 | [8B-8][8B-2] |  | 2,6 | [8B-27][8B-7] [8B-15] | 0 |
| 26 | [8B-8][8B-17] |  | 26 | [8B-8][8B-17] | 0 | 26 | [8B-8][8B-17] [8B-2] |  | 3,5 | [8B-13] | 0 |
| 32 | [8B-24] |  | 40 | [8B-24] | 0 | 40 | [8B-24] |  | 5,2 | [8B-13][8B-7] [8B-15] | [8B-27] |
| 40 | [8B-24] |  | 52 | [8B-7] | 0 | 130 | [8B-8][8B-17] [8B-11][8B-1] |  | 7 | [8B-13] |  |
| 52 | [8B-7] |  | 130 | [8B-8][8B-17] | [8B-7] | 200 | [8B-24] |  | 8 | [8B-21][8B-13] |  |
| 130 | [8B-8][8B-17] [8B-27][8B-7] |  | 200 |  | [8B-24] | 260 | [8B-8][8B-1] | [8B-17][8B-11] | 13 | [8B-13] | [8B-15][8B-7] |
| 200 | [8B-24] |  | 260 | [8B-8] | [8B-17][8B-7] | 400 | 0 | [8B-24] | 20 | [8B-17] | [8B-13] |
| 260 | [8B-8][8B-17][8B-7] |  | 400 |  | [8B-24] | 520 | 0 | [8B-11] | 26 | [8B-25][8B-2][8B-7] | [8B-13] |
| 300 | [8B-24] |  | 520 |  | [8B-17] | 600 | 0 | [8B-24] | 40 |  | [8B-13] |
| 400 |  | [8B-24] | 600 |  | [8B-24] | 800 | 0 | [8B-24] | 52 | [8B-2] |  |
| 520 |  | [8B-17][8B-25] |  |  |  |  |  | 0 | 100 |  | [8B-17] |
| 600 |  | [8B-24] |  |  |  |  |  |  | 200 |  | [8B-17] |
| 800 |  | [8B-24] |  |  |  |  |  |  | 300 |  | [8B-17] |
| 1200 |  | [8B-24] |  |  |  |  |  |  | 400 |  | [8B-17] |
| Sampling rate range [MHz] | 0,96- 10 |  |  | 0,96- 8,64 |  |  | 0,96- 15,36 |  |  | 3,84- 50 |  |
| Inner GB BW range, if applicable [kHz] | 0- 480 |  |  | 120- 480 |  |  | 0- 480 |  |  | N/A |  |
| # of sources with/wo time domain sliding window | [8B-8]/ [8B-24][8B-17][8B-25] [8B-8][8B-7][8B-27] |  |  | [8B-8]/ [8B-24] [8B-17][8B-7] |  |  | [8B-8]/ [8B-24][8B-17][8B-1][8B-8][8B-2][8B-11] |  |  | [-]/ [8B-17][8B-25][8B-13] [8B-21][8B-15][8B-2][8B-27] |  |
| # of sources with/wo frequency adjustment | [8B-8]/ [8B-24][8B-17][8B-25] [8B-8][8B-7][8B-27] |  |  | [8B-8]/ [8B-24] [8B-17][8B-7] |  |  | [8B-8]/ [8B-24][8B-17][8B-1][8B-8][8B-2][8B-11] |  |  | [-]/ [8B-17][8B-25][8B-13] [8B-21][8B-15][8B-2][8B-27] |  |

| Fe [kHz] | OOK-3 |  |
| --- | --- | --- |
|  | <2dB sources | >=2dB sources |
| 9.984 | [8B-15] |  |
| 13 |  | [8B-15] |
| Sampling rate range [MHz] | 30,72- 30,72 |  |
| Inner GB BW range, if applicable [kHz] | N/A |  |
| # of sources with/wo time domain sliding window | [-]/ [8B-15] |  |
| # of sources with/wo frequency adjustment | [-]/ [8B-15] |  |

Frequency error tolerance for the waveform is captured by the following Table 8.3-5:

Table 8.3-5 Frequency tolerance of waveforms

| Waveform | Tolerance up to frequency error [kHz] | Note |
| --- | --- | --- |
| OOK-1 30kHz SCS | 800 | max simulated value |
| OOK-1 60kHz SCS | 400 | max simulated value |
| OOK-1 120kHz SCS | 400 | max simulated value |
| OOK-3 | 9.984 | [8B-15] result |
| OOK-2 M=2 | 300 |  |
| OOK-2 M=4 | 200 | 4 segment OOK-2 is less robust than 2 segment OOK-2 |
| OOK-4 M=2 | 1200 | max simulated value |
| OOK-4 M=4 | 800 | max simulated value |
| OOK-4 M>4 | 390 | max simulated value |
| FSK M=1 | 300 | two segments [8B-8]assumes frequency adjustment which may cause frequency error |
| FSK-1 M=2 | 130 | four segments, and 2 are ON at given time [8B-8] assumes frequency adjustment which may cause frequency error |
| FSK-2 M=2 | 200 | four segments, but only 1 is ON [8B-8] assumes frequency adjustment which may cause frequency error |
| OFDMA | 26 |  |

#### 8.3.2.1A	Observation for frequency error

There are the following observations for frequency error Table 8.3-5:

- Single frequency segment OOK (except OOK3) waveform is more robust to frequency error (of 390 kHz) than OOK/FSK waveforms with multiple frequency segments (depending on guard-band size between segments) and both are more robust than OFDMA waveform assuming no frequency compensation/synchronization.

- [8B-1] showed that single frequency segment FSK2-envelop-IF waveform is more robust to frequency error (of 260 kHz) than OOK/FSK waveforms with multiple frequency segments (depending on guard-band size between segments) and both are more robust than OFDMA waveform assuming no frequency compensation/ synchronization.

### 8.3.3	Spectral efficiency

For the following Waveforms:

- OOK-1 30kHz SCS

- OOK-1 60kHz SCS

- OOK-1 120kHz SCS

- OOK-2 M=2

- OOK-2 M=4

- OOK-3

- OOK-4 M=2

- OOK-4 M=4

- OOK-4 M>4

- FSK M=1

- FSK-1 M=2

- FSK-2 M=2

- OFDMA

With the following assumptions:

- Timing error = 0us.

- Frequency error = 0ppm.

- Only TDL-C results.

- Other parameters are not restricted.

  - table includes results across different receiver types, different power pooling assumption, different sampling rates, different tx antenna configurations, FAR target for the same waveform.

  - best result within a company/source is considered for the table.

Table 8.3-6 presents the results:

Table 8.3-6 Spectral efficiency for different range and waveforms

| OOK-1 30kHz |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SNR Range [dB] | SE median [dB] | SE average [b/s/Hz] | SE Min [b/s/Hz] | SE Max [b/s/Hz] | #Sources [-] |
| [-50, -9] | 0,003241 | 0,003241 | 0,003241 | 0,003241 | [8B-11] |
| [-9, -3] | 0,003241 | 0,003241 | 0,003241 | 0,003241 | [8B-11] [8B-17] |
| [-3, 3] | 0,003241 | 0,00321 | 0,00144 | 0,006481 | [8B-25][8B-3][8B-1][8B-2][8B-7][8B-22][8B-28][8B-8][8B-24][8B-27][8B-11] |
| [3, 9] | 0,003241 | 0,004028 | 0,001389 | 0,01037 | [8B-20][8B-17][8B-1][8B-7][8B-8][8B-27][8B-16] |
| [9, 50] | 0,013889 | 0,016152 | 0,008642 | 0,025926 | [8B-1][8B-2][8B-16] |
| OOK-1 60kHz |  |  |  |  |  |
| SNR Range [dB] | SE 50% percentile [dB] | SE average [b/s/Hz] | SE Min [b/s/Hz] | SE Max [b/s/Hz] | #Sources [-] |
| [-50, -9] | NA |  |  |  |  |
| [-9, -3] | 0,00216 | 0,00216 | 0,00216 | 0,00216 | [8B-21] |
| [-3, 3] | 0,004321 | 0,004295 | 0,000926 | 0,006481 | [8B-21][8B-1][8B-2][8B-7][8B-28][8B-17] |
| [3, 9] | 0,006481 | 0,006327 | 0,003241 | 0,009259 | [8B-3][8B-7][8B-17] |
| [9, 50] | NA |  |  |  | [8B-3][8B-7][8B-17] |
| OOK-1 120kHz |  |  |  |  |  |
| SNR Range [dB] | SE 50% percentile [dB] | SE average [b/s/Hz] | SE Min [b/s/Hz] | SE Max [b/s/Hz] | #Sources [-] |
| [-50, -9] | NA |  |  |  |  |
| [-9, -3] | NA |  |  |  |  |
| [-3, 3] | 0,007562 | 0,007562 | 0,006481 | 0,008642 | [8B-21][8B-28] |
| [3, 9] | 0,010802 | 0,010802 | 0,008642 | 0,012963 | [8B-21][8B-1][8B-2][8B-17] |
| [9, 50] | 0,012963 | 0,012963 | 0,012963 | 0,012963 | [8B-17] |
| OOK-2 M=2 |  |  |  |  |  |
| SNR Range [dB] | SE 50% percentile [dB] | SE average [b/s/Hz] | SE Min [b/s/Hz] | SE Max [b/s/Hz] | #Sources [-] |
| [-50, -9] | NA |  |  |  |  |
| [-9, -3] | 0,012963 | 0,012963 | 0,012963 | 0,012963 | [8B-11] |
| [-3, 3] | 0,006481 | 0,007562 | 0,003241 | 0,012963 | [8B-24][8B-2][8B-11] |
| [3, 9] | 0,00463 | 0,004455 | 0,00144 | 0,006481 | [8B-25][8B-2][8B-7][8B-8][8B-17] |
| [9, 50] | 0,006481 | 0,006481 | 0,006481 | 0,006481 | [8B-8] |
| OOK-2 M=4 |  |  |  |  |  |
| SNR Range [dB] | SE 50% percentile [dB] | SE average [b/s/Hz] | SE Min [b/s/Hz] | SE Max [b/s/Hz] | #Sources [-] |
| [-50, -9] | NA |  |  |  |  |
| [-9, -3] | NA |  |  |  |  |
| [-3, 3] | 0,014583 | 0,014583 | 0,003241 | 0,025926 | [8B-2][8B-11] |
| [3, 9] | 0,012963 | 0,012963 | 0,012963 | 0,012963 | [8B-24][8B-2] |
| [9, 50] | 0,012037 | 0,013014 | 0,006481 | 0,025926 | [8B-1][8B-2][8B-7][8B-22][8B-8][8B-17] |
| OOK-4 M=2 |  |  |  |  |  |
| SNR Range [dB] | SE 50% percentile [dB] | SE average [b/s/Hz] | SE Min [b/s/Hz] | SE Max [b/s/Hz] | #Sources [-] |
| [-50, -9] | NA |  |  |  |  |
| [-9, -3] | 0,002566 | 0,004639 | 0,000463 | 0,012963 | [8B-21][8B-7][8B-27][8B-11] |
| [-3, 3] | 0,004321 | 0,004515 | 0,00144 | 0,006481 | [8B-21][8B-25][8B-3][8B-1][8B-2][8B-7][8B-24][8B-27][8B-17] |
| [3, 9] | 0,006481 | 0,008025 | 0,006481 | 0,011111 | [8B-3][8B-7][8B-27] |
| [9, 50] | NA |  |  |  | [8B-3][8B-7][8B-27] |
| OOK-4 M=4 |  |  |  |  |  |
| SNR Range [dB] | SE 50% percentile [dB] | SE average [b/s/Hz] | SE Min [b/s/Hz] | SE Max [b/s/Hz] | #Sources [-] |
| [-50, -9] | NA |  |  |  |  |
| [-9, -3] | 0,004321 | 0,007121 | 0,000463 | 0,025926 | [8B-13][8B-21][8B-3][8B-2][8B-7][8B-11] |
| [-3, 3] | 0,011111 | 0,010328 | 0,000463 | 0,025926 | [8B-13][8B-21][8B-25][8B-3][8B-2][8B-7][8B-28][8B-24][8B-11] |
| [3, 9] | 0,012037 | 0,010976 | 0,006481 | 0,012963 | [8B-13][8B-3][8B-1][8B-2][8B-7][8B-22][8B-28][8B-17] |
| [9, 50] | 0,019444 | 0,019444 | 0,012963 | 0,025926 | [8B-2][8B-22] |
| OOK-4 M>4 |  |  |  |  |  |
| SNR Range [dB] | SE 50% percentile [dB] | SE average [b/s/Hz] | SE Min [b/s/Hz] | SE Max [b/s/Hz] | #Sources [-] |
| [-50, -9] | NA |  |  |  |  |
| [-9, -3] | 0,009722 | 0,009722 | 0,009722 | 0,009722 | [8B-3] |
| [-3, 3] | 0,015394 | 0,015394 | 0,004861 | 0,025926 | [8B-3][8B-24] |
| [3, 9] | 0,021605 | 0,021605 | 0,017284 | 0,025926 | [8B-3][8B-1] |
| [9, 50] | NA |  |  |  | [8B-3][8B-1] |
| FSK M=1 |  |  |  |  |  |
| SNR Range [dB] | SE 50% percentile [dB] | SE average [b/s/Hz] | SE Min [b/s/Hz] | SE Max [b/s/Hz] | #Sources [-] |
| [-50, -9] | NA |  |  |  |  |
| [-9, -3] | 0,00054 | 0,00054 | 0,00054 | 0,00054 | [8B-27] |
| [-3, 3] | 0,006481 | 0,005401 | 0,003241 | 0,006481 | [8B-24][8B-27][8B-17] |
| [3, 9] | 0,005556 | 0,005118 | 0,002881 | 0,006481 | [8B-25][8B-8][8B-7][8B-27] |
| [9, 50] | 0,006481 | 0,006481 | 0,006481 | 0,006481 | [8B-8] |
| FSK-1 M=2 |  |  |  |  |  |
| SNR Range [dB] | SE 50% percentile [dB] | SE average [b/s/Hz] | SE Min [b/s/Hz] | SE Max [b/s/Hz] | #Sources [-] |
| [-50, -9] | NA |  |  |  |  |
| [-9, -3] | NA |  |  |  |  |
| [-3, 3] | NA |  |  |  |  |
| [3, 9] | 0,012963 | 0,012963 | 0,012963 | 0,012963 | [8B-24][8B-17] |
| [9, 50] | 0,012963 | 0,016667 | 0,011111 | 0,025926 | [8B-8][8B-7][8B-2] |
| FSK-2 M=2 |  |  |  |  |  |
| SNR Range [dB] | SE 50% percentile [dB] | SE average [b/s/Hz] | SE Min [b/s/Hz] | SE Max [b/s/Hz] | #Sources [-] |
| [-50, -9] | NA |  |  |  |  |
| [-9, -3] | 0,008102 | 0,008102 | 0,003241 | 0,012963 | [8B-2][8B-11] |
| [-3, 3] | 0,012963 | 0,012963 | 0,012963 | 0,012963 | [8B-24][8B-2][8B-17] |
| [3, 9] | 0,011111 | 0,010905 | 0,008642 | 0,012963 | [8B-1][8B-7][8B-2] |
| [9, 50] | 0,019444 | 0,019444 | 0,012963 | 0,025926 | [8B-8][8B-2] |
| OFDMA |  |  |  |  |  |
| SNR Range [dB] | SE 50% percentile [dB] | SE average [b/s/Hz] | SE Min [b/s/Hz] | SE Max [b/s/Hz] | #Sources [-] |
| [-50, -9] | 0,025926 | 0,025926 | 0,025926 | 0,025926 | [8B-21] |
| [-9, -3] | 0,012963 | 0,018426 | 0,001768 | 0,077778 | [8B-13][8B-21][8B-25][8B-2][8B-15][8B-27][8B-17] |
| [-3, 3] | 0,012626 | 0,015788 | 0,003241 | 0,038889 | [8B-13][8B-25][8B-2][8B-7][8B-28][8B-24][8B-27][8B-17] |
| [3, 9] | 0,012472 | 0,020819 | 0,006481 | 0,051852 | [8B-13][8B-7][8B-27][8B-17] |
| [9, 50] | NA |  |  |  | [8B-13][8B-7][8B-27][8B-17] |
| OOK-3 |  |  |  |  |  |
| SNR Range [dB] | SE 50% percentile [dB] | SE average [b/s/Hz] | SE Min [b/s/Hz] | SE Max [b/s/Hz] | #Sources [-] |
| [-50, -9] |  |  |  |  |  |
| [-9, -3] | 0.004 | 0.004 | 0.004 | 0.004 | [8B-15] |
| [-3, 3] |  |  |  |  |  |
| [3, 9] |  |  |  |  |  |
| [9, 50] |  |  |  |  |  |

Note 1: The spectral efficiency of OFDMA waveform is dependent on the selection of sequences

Note 2: Spectral efficiency: SE = LP-WUS information size [bits]/ LP-WUS length [s] / LP-WUS BW [Hz]

### 8.3.4	RSRP RRM measurement accuracy

#### 8.3.4.1	Results for LP-SS RSRP RRM measurement, TDL-C

Given assumption:

- Resource unit is defined as total resource used for measurement, i.e.  resource length [sym] * # of samples used for averaging.

- X is the SNR target for measurement, Y is the measurement accuracy (delta-RSRP for 90% measurements)

- Results are shown per each value of X in particular Resource unit range.

- Impairments considered are shown in the table.

- TDL-C 300 channel

- X is SNR observed by LP-WUR. Corresponding SNR observed by MR depends on e.g. NF difference between MR and LR

Results are summarised in Table 8.3-7

Table 8.3-7 RSRP TDL-C channel RSRP accuracy

| Resource unit [sym] range 1-10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X=-9 Y=-> | 10 | 3 | 10 | 3 | 10 |  |  |  |
| Source | [8B-25] | [8B-25] | [8B-25] | [8B-25] | [8B-25] |  |  |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:0 | Te:0 Fe:50 | Te:0 Fe:50 | Te:2 Fe:0 |  |  |  |
| # samples | 1 | 2 | 1 | 2 | 2 |  |  |  |
| X=-6 Y=-> | 1,01 |  |  |  |  |  |  |  |
| Source | [8B-13] |  |  |  |  |  |  |  |
| Impairment | Te:0 Fe:0 |  |  |  |  |  |  |  |
| # samples | 1 |  |  |  |  |  |  |  |
| X=-3 Y=-> | 1,5 | 0,8 | 1,5 | 0,8 | 5 | 2,3 | 2,25 |  |
| Source | [8B-25] | [8B-25] | [8B-25] | [8B-25] | [8B-25] | [8B-25] | [8B-1] |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:0 | Te:0 Fe:50 | Te:0 Fe:50 | Te:2 Fe:0 | Te:2 Fe:0 | Te:0 Fe:0 |  |
| # samples | 1 | 2 | 1 | 2 | 1 | 2 | 1 |  |
| Resource unit [sym] range 11-20 |  |  |  |  |  |  |  |  |
| X=-11 Y=-> | 7,4 | 7,51 | 7,58 |  |  |  |  |  |
| Source | [8B-7] | [8B-7] | [8B-7] |  |  |  |  |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:200 | Te:4 Fe:0 |  |  |  |  |  |
| # samples | 1 | 1 | 1 |  |  |  |  |  |
| X=-9 Y=-> | 2 | 2 | 7 | 3,5 | 6,87 | 7,07 | 7,16 | 1,38 |
| Source | [8B-25] | [8B-25] | [8B-25] | [8B-25] | [8B-7] | [8B-7] | [8B-7] | [8B-13] |
| Impairment | Te:0 Fe:0 | Te:0 Fe:50 | Te:2 Fe:0 | Te:0 Fe:0 | Te:0 Fe:0 | Te:0 Fe:200 | Te:4 Fe:0 | Te:0 Fe:0 |
| # samples | 3 | 3 | 3 | 1 | 1 | 1 | 1 | 1 |
| X=-6 Y=-> | 5,93 | 6,12 | 6,29 | 0,78 |  |  |  |  |
| Source | [8B-7] | [8B-7] | [8B-7] | [8B-13] |  |  |  |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:200 | Te:4 Fe:0 | Te:0 Fe:0 |  |  |  |  |
| # samples | 1 | 1 | 1 | 1 |  |  |  |  |
| X=-3 Y=-> | 0,5 | 0,5 | 1,8 |  |  |  |  |  |
| Source | [8B-25] | [8B-25] | [8B-25] |  |  |  |  |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:50 | Te:2 Fe:0 |  |  |  |  |  |
| # samples | 3 | 3 | 3 |  |  |  |  |  |
| Resource unit [sym] range 21-30 |  |  |  |  |  |  |  |  |
| X=-11 Y=-> | 4,82 | 5,39 | 5,44 |  |  |  |  |  |
| Source | [8B-7] | [8B-7] | [8B-7] |  |  |  |  |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:200 | Te:4 Fe:0 |  |  |  |  |  |
| # samples | 2 | 2 | 2 |  |  |  |  |  |
| X=-9 Y=-> | 2,8 | 4,93 | 5,08 | 5,15 |  |  |  |  |
| Source | [8B-25] | [8B-7] | [8B-7] | [8B-7] |  |  |  |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:0 | Te:0 Fe:200 | Te:4 Fe:0 |  |  |  |  |
| # samples | 1 | 2 | 2 | 2 |  |  |  |  |
| X=-6 Y=-> | 4,27 | 4,4 | 4,58 |  |  |  |  |  |
| Source | [8B-7] | [8B-7] | [8B-7] |  |  |  |  |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:200 | Te:4 Fe:0 |  |  |  |  |  |
| # samples | 2 | 2 | 2 |  |  |  |  |  |
| Resource unit [sym] range 31-50 |  |  |  |  |  |  |  |  |
| X=-11 Y=-> | 4,34 | 4,41 | 4,46 |  |  |  |  |  |
| Source | [8B-7] | [8B-7] | [8B-7] |  |  |  |  |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:200 | Te:4 Fe:0 |  |  |  |  |  |
| # samples | 3 | 3 | 3 |  |  |  |  |  |
| X=-9 Y=-> | 1,8 | 4,05 | 4,15 | 4,23 |  |  |  |  |
| Source | [8B-25] | [8B-7] | [8B-7] | [8B-7] |  |  |  |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:0 | Te:0 Fe:200 | Te:4 Fe:0 |  |  |  |  |
| # samples | 1 | 3 | 3 | 3 |  |  |  |  |
| X=-6 Y=-> | 3,51 | 3,62 | 3,75 |  |  |  |  |  |
| Source | [8B-7] | [8B-7] | [8B-7] |  |  |  |  |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:200 | Te:4 Fe:0 |  |  |  |  |  |
| # samples | 3 | 3 | 3 |  |  |  |  |  |
| Resource unit [sym] range 51-70 |  |  |  |  |  |  |  |  |
| X=-11 Y=-> | 3,78 | 3,36 | 3,83 | 3,37 | 3,87 | 3,48 |  |  |
| Source | [8B-7] | [8B-7] | [8B-7] | [8B-7] | [8B-7] | [8B-7] |  |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:0 | Te:0 Fe:200 | Te:0 Fe:200 | Te:4 Fe:0 | Te:4 Fe:0 |  |  |
| # samples | 4 | 5 | 4 | 5 | 4 | 5 |  |  |
| X=-9 Y=-> | 3,53 | 3,13 | 3,6 | 3,18 | 3,65 | 3,28 | 0,57 |  |
| Source | [8B-7] | [8B-7] | [8B-7] | [8B-7] | [8B-7] | [8B-7] | [8B-13] |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:0 | Te:0 Fe:200 | Te:0 Fe:200 | Te:4 Fe:0 | Te:4 Fe:0 | Te:0 Fe:0 |  |
| # samples | 4 | 5 | 4 | 5 | 4 | 5 | 5 |  |
| X=-6 Y=-> | 3,05 | 2,71 | 3,14 | 2,78 | 3,27 | 2,92 | 0,32 |  |
| Source | [8B-7] | [8B-7] | [8B-7] | [8B-7] | [8B-7] | [8B-7] | [8B-13] |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:0 | Te:0 Fe:200 | Te:0 Fe:200 | Te:4 Fe:0 | Te:4 Fe:0 | Te:0 Fe:0 |  |
| # samples | 4 | 5 | 4 | 5 | 4 | 5 | 5 |  |

#### 8.3.4.1A	Observation RSRP LP-SS, TDL-C:

For OOK based LP-SS RSRP measurement accuracy ([8B-1][8B-13],[8B-7],[8B-25])

- at SNR X=-6, to achieve 90 % accuracy of measurement +-Y<=3dB,  [8B-13],[8B-7] show 51-70 symbols spread over 4-5 periods of LP-SS are sufficient,  [8B-13] shows 11-20 symbols spread over 1 period is sufficient.

- at SNR X=-3, to achieve 90 % accuracy of measurement +-Y<=3dB, less than 10 symbols spread over 1-2 periods is sufficient. [8B-25][8B-1]

- at SNR X=-9, to achieve 90 % accuracy of measurement +-Y<=5dB, [8B-25],[8B-7] show 21-30 symbols spread over 4-5 periods of LP-SS are sufficient, [8B-13] shows that 11-20 symbols within 1 period are sufficient

- at SNR X=-6, to achieve 90 % accuracy of measurement +-Y<=5dB,.  [8B-7] shows 21-30 symbols spread over 4-5 periods of LP-SS are sufficient, [8B-13] shows that 11-20 symbols within 1 period are sufficient.

#### 8.3.4.2	Results for LP-SS RSRP RRM measurement, AWGN

Given assumptions:

- Resource unit is defined as total resource used for measurement, i.e., resource length [sym] * # of samples used for averaging.

- X is the SNR target for measurement, Y is the measurement accuracy (delta-RSRP for 90% measurements)

- Results are shown per each value of X in particular Resource unit range.

- Impairments and Sources are shown as well.

- AWGN channel.

Results are summarised in Table 8.3-8

Table 8.3-8 RSRP AWGN channel RSRP accuracy

| Resource unit [sym] range 1-10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| X=-9 Y=-> | 3 | 2 |  |  |  |
| Source | [8B-25] | [8B-25] |  |  |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:0 |  |  |  |
| # samples | 1 | 2 |  |  |  |
| X=-6 Y=-> | 0,81 |  |  |  |  |
| Source | [8B-13] |  |  |  |  |
| Impairment | Te:0 Fe:0 |  |  |  |  |
| # samples | 1 |  |  |  |  |
| X=-3 Y=-> | 0,75 | 0,55 |  |  |  |
| Source | [8B-25] | [8B-25] |  |  |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:0 |  |  |  |
| # samples | 1 | 2 |  |  |  |
| Resource unit [sym] range 11-20 |  |  |  |  |  |
| X=-11 Y=-> | 3,89 | 4,25 | 4,32 |  |  |
| Source | [8B-7] | [8B-7] | [8B-7] |  |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:200 | Te:4 Fe:0 |  |  |
| # samples | 1 | 1 | 1 |  |  |
| X=-9 Y=-> | 1,7 | 2,22 | 2,73 | 2,78 | 1,17 |
| Source | [8B-25] | [8B-7] | [8B-7] | [8B-7] | [8B-13] |
| Impairment | Te:0 Fe:0 | Te:0 Fe:0 | Te:0 Fe:200 | Te:4 Fe:0 | Te:0 Fe:0 |
| # samples | 3 | 1 | 1 | 1 | 1 |
| X=-6 Y=-> | 1,15 | 1,3 | 1,33 | 0,64 |  |
| Source | [8B-7] | [8B-7] | [8B-7] | [8B-13] |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:200 | Te:4 Fe:0 | Te:0 Fe:0 |  |
| # samples | 1 | 1 | 1 | 1 |  |
| X=-3 Y=-> | 0,5 |  |  |  |  |
| Source | [8B-25] |  |  |  |  |
| Impairment | Te:0 Fe:0 |  |  |  |  |
| # samples | 3 |  |  |  |  |
| Resource unit [sym] range 21-30 |  |  |  |  |  |
| X=-11 Y=-> | 3,28 | 3,21 | 3,28 |  |  |
| Source | [8B-7] | [8B-7] | [8B-7] |  |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:200 | Te:4 Fe:0 |  |  |
| # samples | 2 | 2 | 2 |  |  |
| Resource Range unit | 30 | 50 |  |  |  |
| X=-6 Y=-> | 0,38 |  |  |  |  |
| Source | [8B-13] |  |  |  |  |
| Impairment | Te:0 Fe:0 |  |  |  |  |
| # samples | 5 |  |  |  |  |
| Resource unit [sym] range 51-70 |  |  |  |  |  |
| X=-9 Y=-> | 0,52 |  |  |  |  |
| Source | [8B-13] |  |  |  |  |
| Impairment | Te:0 Fe:0 |  |  |  |  |
| # samples | 5 |  |  |  |  |
| X=-6 Y=-> | 0,3 |  |  |  |  |
| Source | [8B-13] |  |  |  |  |
| Impairment | Te:0 Fe:0 |  |  |  |  |
| # samples | 5 |  |  |  |  |

#### 8.3.4.2A	Observation RSRP LP-SS, AWGN:

For OOK based LP-SS RSRP measurement accuracy ( [8B-13],[8B-7][8B-25]),

- at SNR X=-9, to achieve 90 % accuracy of measurement +-Y<=3dB, <=10 OFDMA symbols over 1-3 periods are sufficient.

- at SNR X=-11, to achieve 90 % accuracy of measurement +-Y<=5dB, 11-20 OFDMA symbols over 1-3 periods are sufficient.

#### 8.3.4.3	Results for SSS RSRP RRM measurement, TDL-C

Given assumptions:

- Resource unit is defined as total resource used for measurement, i.e. resource length [sym] * # of samples used for averaging.

- X is the SNR target for measurement, Y is the measurement accuracy (delta-RSRP for 90% measurements)

- Results are shown per each value of X in particular Resource unit range.

- Impairments and Sources are shown as well.

- Only TDL-C 300 results

Results are summarised in Table 8.3-9

Table 8.3-9 RSRP SSS TDL-C

| Resource unit [sym] 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| X=-6 Y=-> | 1,5 | 1,85 | 1,8 |  |  |  |
| Source | [8B-25] | [8B-25] | [8B-25] |  |  |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:10 | Te:2 Fe:0 |  |  |  |
| # samples | 1 | 1 | 1 |  |  |  |
| X=-3 Y=-> | 0,95 | 1,25 | 1,3 | 1,6443 | 1,7817 | 2,2314 |
| Source | [8B-25] | [8B-25] | [8B-25] | [8B-27] | [8B-27] | [8B-27] |
| Impairment | Te:0 Fe:0 | Te:0 Fe:10 | Te:2 Fe:0 | Te:0 Fe:0 | Te:0 Fe:1 | Te:0 Fe:2 |
| # samples | 1 | 1 | 1 | 1 | 1 | 1 |
| Resource unit [sym] 2 |  |  |  |  |  |  |
| X=-6 Y=-> | 1,2 | 1,4 | 1,3 |  |  |  |
| Source | [8B-25] | [8B-25] | [8B-25] |  |  |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:10 | Te:2 Fe:0 |  |  |  |
| # samples | 2 | 2 | 2 |  |  |  |
| X=-3 Y=-> | 0,7 | 0,85 | 0,9 | 1,0021 | 1,0608 | 1,5672 |
| Source | [8B-25] | [8B-25] | [8B-25] | [8B-27] | [8B-27] | [8B-27] |
| Impairment | Te:0 Fe:0 | Te:0 Fe:10 | Te:2 Fe:0 | Te:0 Fe:0 | Te:0 Fe:1 | Te:0 Fe:2 |
| # samples | 2 | 2 | 2 | 2 | 2 | 2 |
| Resource unit [sym] 3 |  |  |  |  |  |  |
| X=-6 Y=-> | 0,9 | 1,1 | 1,1 |  |  |  |
| Source | [8B-25] | [8B-25] | [8B-25] |  |  |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:10 | Te:2 Fe:0 |  |  |  |
| # samples | 3 | 3 | 3 |  |  |  |
| X=-3 Y=-> | 0,5 | 0,65 | 0,65 |  |  |  |
| Source | [8B-25] | [8B-25] | [8B-25] |  |  |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:10 | Te:2 Fe:0 |  |  |  |
| # samples | 3 | 3 | 3 |  |  |  |
| Resource unit [sym] 4 |  |  |  |  |  |  |
| X=-3 Y=-> | 0,6481 | 0,7332 | 1,2474 |  |  |  |
| Source | [8B-27] | [8B-27] | [8B-27] |  |  |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:1 | Te:0 Fe:2 |  |  |  |
| # samples | 4 | 4 | 4 |  |  |  |
| Resource unit [sym] 5 |  |  |  |  |  |  |
| X=-3 Y=-> | 0,6195 | 0,7151 | 1,0843 |  |  |  |
| Source | [8B-27] | [8B-27] | [8B-27] |  |  |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:1 | Te:0 Fe:2 |  |  |  |
| # samples | 5 | 5 | 5 |  |  |  |

#### 8.3.4.3A	Observation for SSS RSRP RRM measurement, TDL-C

For SSS based RSRP RRM measurement accuracy ([8B-27][8B-25])

- at SNR X=-6, to achieve 90 % accuracy of measurement +-Y<=3dB, 1 symbol is sufficient

- at SNR X=-3, to achieve 90 % accuracy of measurement +-Y<=3dB, 1 symbol is sufficient

### 8.3.5	RSRQ RRM measurement accuracy

#### 8.3.5.1	Results LP-SS, AWGN

Given assumptions:

- Resource unit is defined as total resource used for measurement, i.e. resource length [sym] * # of samples used for averaging.

- X is the SNR target for measurement, Y is the measurement accuracy (delta-RSRQ for 90% measurements)

- Results are shown per each value of X in particular Resource unit range.

- Impairments and Sources are shown as well.

- AWGN channel

The results are summarized in Table 8.3-10:

Table 8.3-10 RSRQ measurement accuracy LP-SS, AWGN

| Resource unit [sym] range 11-20 |  |  |  |
| --- | --- | --- | --- |
| X=-11 Y=-> | 3,87 | 4,23 | 4,3 |
| Source | [8B-7] | [8B-7] | [8B-7] |
| Impairment | Te:0 Fe:0 | Te:0 Fe:200 | Te:4 Fe:0 |
| # samples | 1 | 1 | 1 |
| X=-9 Y=-> | 2,2 | 2,35 | 2,39 |
| Source | [8B-7] | [8B-7] | [8B-7] |
| Impairment | Te:0 Fe:0 | Te:0 Fe:200 | Te:4 Fe:0 |
| # samples | 1 | 1 | 1 |
| X=-6 Y=-> | 1,13 | 1,19 | 1,21 |
| Source | [8B-7] | [8B-7] | [8B-7] |
| Impairment | Te:0 Fe:0 | Te:0 Fe:200 | Te:4 Fe:0 |
| # samples | 1 | 1 | 1 |
| Resource Range unit | 20 | 30 |  |
| X=-11 Y=-> | 2,93 | 3,2 | 3,26 |
| Source | [8B-7] | [8B-7] | [8B-7] |
| Impairment | Te:0 Fe:0 | Te:0 Fe:200 | Te:4 Fe:0 |
| # samples | 2 | 2 | 2 |

#### 8.3.5.1A	Observation RSRQ LP-SS, AWGN:

For OOK-based LP-SS. RSRQ measurement accuracy ([8B-7])

- at SNR X=-6, to achieve 90 % accuracy of measurement +-Y<=3dB, 11-20 OFDMA symbols over 1 period are sufficient.

- at SNR X=-6, to achieve 90 % accuracy of measurement +-Y<=5dB, 11-20 OFDMA symbols over 1 period are sufficient.

- at SNR X=-9, to achieve 90 % accuracy of measurement +-Y<=5dB, 11-20 OFDMA symbols over 1 period are sufficient.

#### 8.3.5.2	Results LP-SS, TDL-C

Given assumptions:

- Resource unit is defined as total resource used for measurement, i.e. resource length [sym] * # of samples used for averaging.

- X is the SNR target for measurement, Y is the measurement accuracy (delta-RSRQ for 90% measurements)

- Results are shown per each value of X in particular Resource unit range.

- Impairments and Sources are shown as well.

- TDL-C channel

The results are summarized in Table 8.3-11:

Table 8.3-11 RSRQ measurement accuracy LP-SS, TDL-C

| Resource unit [sym] range 11-20 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| X=-11 Y=-> | 7,14 | 7,27 | 7,33 |  |  |  |
| Source | [8B-7] | [8B-7] | [8B-7] |  |  |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:200 | Te:4 Fe:0 |  |  |  |
| # samples | 1 | 1 | 1 |  |  |  |
| X=-9 Y=-> | 6,45 | 6,67 | 6,74 |  |  |  |
| Source | [8B-7] | [8B-7] | [8B-7] |  |  |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:200 | Te:4 Fe:0 |  |  |  |
| # samples | 1 | 1 | 1 |  |  |  |
| X=-6 Y=-> | 5,13 | 5,35 | 5,51 |  |  |  |
| Source | [8B-7] | [8B-7] | [8B-7] |  |  |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:200 | Te:4 Fe:0 |  |  |  |
| # samples | 1 | 1 | 1 |  |  |  |
| Resource unit [sym] range 20-30 |  |  |  |  |  |  |
| X=-11 Y=-> | 5,16 | 5,23 | 5,27 |  |  |  |
| Source | [8B-7] | [8B-7] | [8B-7] |  |  |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:200 | Te:4 Fe:0 |  |  |  |
| # samples | 2 | 2 | 2 |  |  |  |
| X=-9 Y=-> | 4,65 | 4,81 | 4,88 |  |  |  |
| Source | [8B-7] | [8B-7] | [8B-7] |  |  |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:200 | Te:4 Fe:0 |  |  |  |
| # samples | 2 | 2 | 2 |  |  |  |
| X=-6 Y=-> | 3,75 | 3,89 | 4,05 |  |  |  |
| Source | [8B-7] | [8B-7] | [8B-7] |  |  |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:200 | Te:4 Fe:0 |  |  |  |
| # samples | 2 | 2 | 2 |  |  |  |
| Resource unit [sym] range 30-50 |  |  |  |  |  |  |
| X=-11 Y=-> | 4,21 | 4,29 | 4,33 |  |  |  |
| Source | [8B-7] | [8B-7] | [8B-7] |  |  |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:200 | Te:4 Fe:0 |  |  |  |
| # samples | 3 | 3 | 3 |  |  |  |
| X=-9 Y=-> | 3,84 | 3,94 | 4,02 |  |  |  |
| Source | [8B-7] | [8B-7] | [8B-7] |  |  |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:200 | Te:4 Fe:0 |  |  |  |
| # samples | 3 | 3 | 3 |  |  |  |
| X=-6 Y=-> | 3,08 | 3,22 | 3,34 |  |  |  |
| Source | [8B-7] | [8B-7] | [8B-7] |  |  |  |
| Impairment | Te:0 Fe:0 | Te:0 Fe:200 | Te:4 Fe:0 |  |  |  |
| # samples | 3 | 3 | 3 |  |  |  |
| Resource unit [sym] range 50-70 |  |  |  |  |  |  |
| X=-11 Y=-> | 3,68 | 3,27 | 3,72 | 3,29 | 3,77 | 3,39 |
| Source | [8B-7] | [8B-7] | [8B-7] | [8B-7] | [8B-7] | [8B-7] |
| Impairment | Te:0 Fe:0 | Te:0 Fe:0 | Te:0 Fe:200 | Te:0 Fe:200 | Te:4 Fe:0 | Te:4 Fe:0 |
| # samples | 4 | 5 | 4 | 5 | 4 | 5 |
| X=-9 Y=-> | 3,35 | 2,96 | 3,43 | 3,03 | 3,47 | 3,12 |
| Source | [8B-7] | [8B-7] | [8B-7] | [8B-7] | [8B-7] | [8B-7] |
| Impairment | Te:0 Fe:0 | Te:0 Fe:0 | Te:0 Fe:200 | Te:0 Fe:200 | Te:4 Fe:0 | Te:4 Fe:0 |
| # samples | 4 | 5 | 4 | 5 | 4 | 5 |
| X=-6 Y=-> | 2,69 | 2,38 | 2,81 | 2,48 | 2,94 | 2,62 |
| Source | [8B-7] | [8B-7] | [8B-7] | [8B-7] | [8B-7] | [8B-7] |
| Impairment | Te:0 Fe:0 | Te:0 Fe:0 | Te:0 Fe:200 | Te:0 Fe:200 | Te:4 Fe:0 | Te:4 Fe:0 |
| # samples | 4 | 5 | 4 | 5 | 4 | 5 |

#### 8.3.5.2A	Observation RSRQ LP-SS, TDL-C:

For OOK-based LP-SS measurement accuracy ( [8B-7])

- at SNR X=-6, to achieve 90 % accuracy of measurement +-Y<=3dB, 51-70 symbols over 4 periods are sufficient.

- at SNR X=-6, to achieve 90 % accuracy of measurement +-Y<=5dB, 21-30 symbols over 2 periods are sufficient.

- at SNR X=-9, to achieve 90 % accuracy of measurement +-Y<=5dB, 21-30 symbols over 2 periods are sufficient.

## 8.4	Network power consumption

### 8.4.1	Collection of the results

Evaluation of the network power consumption for introducing LP-SS is summarized in Table 8.4.

It is assumed that the LP-SS periodicity is 320ms. And for more results and details, it can be found in Annex 8.4.

Table 8.4 Summary of network power consumption increases for introducing LP-SS

| Network load | Company | LP-SS periodicity | LP-SS duration/BW | beam | Additional NW power consumption |
| --- | --- | --- | --- | --- | --- |
| zero | [8A-11] | 320ms | 14symbols per beam, 8 slots in total and 11PRBs | 8 | 1.097%~3.55% |
|  | [8A-13] | 320ms | 7symbols per beam, 4slots in total and 12 PRBs | 4 | 0.06%~0.50% |
|  | [8A-5] | 320ms | 28symbols per beam,5Mhz | 4 | 0.122% |
|  | [8A-3] | 320ms | 14 symbol duration (1 slot) per beam, 5MHz | 8 | 3.9% |
|  |  | 320ms | 42 symbol duration per beam, 5MHz | 8 | 11.4% |
|  |  | 320ms | 4 symbol duration per beam, 5MHz | 8 | 1.1% |
| low | [8A-11] | 320ms | 14symbols per beam, 8 slots in total and 11PRBs | 8 | 0.912%~2.716% |
|  | [8A-13] | 320ms | 7symbols per beam, 4slots in total and 12 PRBs | 4 | 0.07%~0.7% |
| Medium | [8A-11] | 320ms | 14symbols per beam, 8 slots in total and 11PRBs | 8 | 0.388%~1.076% |

Note: the definition of network load is given in TR 38.864.

For evaluation purpose, - a load (L) % of a cell is a percentage of resources used for UE specific PDSCH / PUSCH - The following load scenarios are considered Load scenario; Characteristics. Idle/empty load; - Include cell-specific signals and channels, and - L = 0. low load; - Include cell-specific signals and channels, and - 0 < L≤15. Light load; - Include cell-specific signals and channels, and - 15 < L≤ 30. Medium load; - Include cell-specific signals and channels, and - 30 < L≤ 50. For CA, the companies report whether the load is defined per CC or across all CCs..

### 8.4.2	Observations

When 320ms LP-SS periodicity, 4 or 8 beams and no more than 14 symbols LP-SS duration is assumed,

- For zero load, results in [8A-3] [8A-11] [8A-13] show that additional increased network power consumption rate is (0.06%~3.9%);

- For low load, results in [8A-11] [8A-13] show that additional increased network power consumption rate is (0.07%~2.716%);

- For medium load, results in [8A-11] show that additional increased network power consumption rate is (0.388%~1.076%).

When 320ms LP-SS periodicity, 4 beams and 28 symbols LP-SS duration is assumed,

- For zero load, results in [8A-5] show that additional increased network power consumption rate is 0.122%;

When 320ms LP-SS periodicity, 8 beams and 42 symbols LP-SS duration is assumed,

- For zero load, results in [8A-3] show that additional increased network power consumption rate is 11.4%;

Lower impact to the network power consumption is expected when LP-SS is transmitted FDM with NR SSB/SIB-1.

LP-SS is assumed to be additional signal transmission than the existing NR signal/channels.

## 8.5	System overhead

### 8.5.1	Collection of the results

Evaluation of the LP-WUS and LP-SS overhead are summarized in Table 8.5. And for more results and details, it can be found in Annex 8.5.

#### 8.5.1.1	RRC IDLE/INACTIVE

##### 8.5.1.1.1	LP-WUS

The System overhead of LP-WUS for RRC IDLE/INACTIVE is summarized in Table 8.5.1-1

Table 8.5.1 - 1 Summary of System overhead for LP-WUS for RRC IDLE/INACTIVE

| Source | Signal | # of info bits carried by signal | Overhead (%) | T resources per beam | F resources | SCS | number of paged UE per second | Number of LP-WUS transmissions per second | System configuration, e.g., BW and etc. | the target channel | LP-WUS MIL | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [8A-11] | LP-WUS | 1 | 0.0150% | 1 symbol | 5.04MHz | 30kHz | 10 | 10 | 100MHz, 8beams | Msg.3 | 144.86 | For IoT, cases, Assuming 1,000,000 user/km2 connection density [ITU M.2412], with packet arrival every 1 message/2 hours/device [ITU M.2412], assuming 500m ISD with 3 sectors. The coverage of LP-WUS with 1/4/5 symbols is comparable to or better than Msg 3 coverage. OOK LP-WUS is assumed. |
| [8A-11] | LP-WUS | 1 | 0.0030% | 1 symbol | 5.04MHz | 30kHz | 10 | 3 | 100MHz, 8beams | Msg.3 | 144.86 |  |
| [8A-11] | LP-WUS | 8 | 0.0600% | 4 symbols | 5.04MHz | 30kHz | 10 | 10 | 100MHz, 8beams | Msg.3 | 144.37 |  |
| [8A-11] | LP-WUS | 8 | 0.0100% | 4 symbols | 5.04MHz | 30kHz | 10 | 3 | 100MHz, 8beams | Msg.3 | 144.37 |  |
| [8A-11] | LP-WUS | 8 or 12 | 0.0750% | 5 symbols | 5.04MHz | 30kHz | 10 | 10 | 100MHz, 8beams | Msg.3 | 144.05 |  |
| [8A-11] | LP-WUS | 8 or 12 | 0.0130% | 5 symbols | 5.04MHz | 30kHz | 10 | 3 | 100MHz, 8beams | Msg.3 | 144.05 |  |
| [8A-11] | LP-WUS | 1 | 0.0730% | 1 symbol | 5.04MHz | 30kHz | 10 | 10 | 20MHz, 8beams | Msg.3 | 144.86 |  |
| [8A-11] | LP-WUS | 1 | 0.0180% | 1 symbol | 5.04MHz | 30kHz | 10 | 3 | 20MHz, 8beams | Msg.3 | 144.86 |  |
| [8A-11] | LP-WUS | 8 | 0.2900% | 4 symbols | 5.04MHz | 30kHz | 10 | 10 | 20MHz, 8beams | Msg.3 | 144.37 |  |
| [8A-11] | LP-WUS | 8 | 0.0700% | 4 symbols | 5.04MHz | 30kHz | 10 | 3 | 20MHz, 8beams | Msg.3 | 144.37 |  |
| [8A-11] | LP-WUS | 8 or 12 | 0.3630% | 5 symbols | 5.04MHz | 30kHz | 10 | 10 | 20MHz, 8beams | Msg.3 | 144.05 |  |
| [8A-11] | LP-WUS | 8 or 12 | 0.0880% | 5 symbols | 5.04MHz | 30kHz | 10 | 3 | 20MHz, 8beams | Msg.3 | 144.05 |  |
| [8A-11] | LP-WUS | 1 | 0.0550% | 1 symbol | 5.04MHz | 30kHz | 39 | 39 | 100MHz, 8beams | Msg.3 | 144.86 | For eMBB cases, Assuming very high population density, e.g., sports event with 5,000 users/cell population density, and per UE paging arrival rate = 1% when paging i-DRX cycle is Tpagingcycle = 1.28s The coverage of LP-WUS with 1/4/5 symbols is comparable to or better than Msg 3 coverage. OOK LP-WUS is assumed. |
| [8A-11] | LP-WUS | 1 | 0.0150% | 1 symbol | 5.04MHz | 30kHz | 39 | 10 | 100MHz, 8beams | Msg.3 | 144.86 |  |
| [8A-11] | LP-WUS | 8 | 0.2200% | 4 symbols | 5.04MHz | 30kHz | 39 | 39 | 100MHz, 8beams | Msg.3 | 144.37 |  |
| [8A-11] | LP-WUS | 8 | 0.0600% | 4 symbols | 5.04MHz | 30kHz | 39 | 10 | 100MHz, 8beams | Msg.3 | 144.37 |  |
| [8A-11] | LP-WUS | 8 or 12 | 0.2750% | 5 symbols | 5.04MHz | 30kHz | 39 | 39 | 100MHz, 8beams | Msg.3 | 144.05 |  |
| [8A-11] | LP-WUS | 8 or 12 | 0.0750% | 5 symbols | 5.04MHz | 30kHz | 39 | 10 | 100MHz, 8beams | Msg.3 | 144.05 |  |
| [8A-11] | LP-WUS | 1 | 0.2800% | 1 symbol | 5.04MHz | 30kHz | 39 | 39 | 20MHz, 8beams | Msg.3 | 144.86 |  |
| [8A-11] | LP-WUS | 1 | 0.0700% | 1 symbol | 5.04MHz | 30kHz | 39 | 10 | 20MHz, 8beams | Msg.3 | 144.86 |  |
| [8A-11] | LP-WUS | 8 | 1.1200% | 4 symbols | 5.04MHz | 30kHz | 39 | 39 | 20MHz, 8beams | Msg.3 | 144.37 |  |
| [8A-11] | LP-WUS | 8 | 0.2800% | 4 symbols | 5.04MHz | 30kHz | 39 | 10 | 20MHz, 8beams | Msg.3 | 144.37 |  |
| [8A-11] | LP-WUS | 8 or 12 | 1.4000% | 5 symbols | 5.04MHz | 30kHz | 39 | 39 | 20MHz, 8beams | Msg.3 | 144.05 |  |
| [8A-11] | LP-WUS | 8 or 12 | 0.3500% | 5 symbols | 5.04MHz | 30kHz | 39 | 10 | 20MHz, 8beams | Msg.3 | 144.05 |  |
| [8A-13] | LP-WUS | 8 | <1% | 7 or 14 symbols | 5MHz | 15KHz/30kHz | Paging rate:1%,0.1%,0.01%,0.001%; POs number per PF: 1,2,4; PFs number per cycle: 1,2,4,8,16 |  | 20M, 100M; Beams:1,4 | Msg.3 | For LP-WUS with 7 symbols, LP-WUS MIL is 152.28; For LP-WUS with 14 symbols, LP-WUS MIL is 156.94 | Only one case when 20M system bandwidth, 4beams, no grouping, 15KHz, 14OFDM symbols, paging rate 1%, maximum POs per PF, maximum PFs per cycle are configured, the system overhead may reach 1.6%. OOK LP-WUS is assumed. |
| [8A-5] | LP-WUS+LP-SS | 24 | 0.1230% | 12 symbols (no repetition) | 5MHz | 30KHz | 50 paging per second | 50 | 100M, 1 beam |  | Sequence modulation w/o repetition: 149.06 Sequence on top of OOK with 3dB power boosting: 149.36 | Resource overhead for 56kbps data rate, which corresponds to the following waveforms to match comparable Msg3 MIL: sequence modulation w/o repetition Sequence on top of OOK with 3dB power boosting |
| [8A-5] | LP-WUS+LP-SS | 24 | 0.2300% | 24 symbols for 2 repetitions | 5MHz | 30KHz | 50 paging per second | 50 | 100M, 1 beam |  | FSK-2 transmission with 2 repetitions: 149.18 Sequence on top of OOK/FSK with 2 repetitions: 151 | Resource overhead for 28kbps data rate, which corresponds to the following waveforms to match comparable Msg3 MIL: FSK-2 transmission with 2 repetitions Sequence on top of OOK/FSK with 2 repetitions |
| [8A-5] | LP-WUS+LP-SS | 24 | 0.4440% | 48 symbols for 4 repetitions | 5MHz | 30KHz | 50 paging per second | 50 | 100M, 1 beam |  | OOK transmission with 4 repetitions: 148.68 | Resource overhead for 14kbps data rate, which corresponds to the following waveforms to match comparable Msg3 MIL: OOK transmission with 4 repetitions |
| [8A-5] | LP-WUS+LP-SS | 24 | 0.4910% | 12 symbols (no repetition) | 5MHz | 30KHz | 50 paging per second | 50 | 100M, 4 beams |  | Sequence modulation w/o repetition: 149.06 Sequence on top of OOK with 3dB power boosting: 149.36 | Resource overhead for 56kbps data rate, which corresponds to the following waveforms to match comparable Msg3 MIL: sequence modulation w/o repetition Sequence on top of OOK with 3dB power boosting |
| [8A-5] | LP-WUS+LP-SS | 24 | 0.9200% | 24 symbols for 2 repetitions | 5MHz | 30KHz | 50 paging per second | 50 | 100M, 4 beams |  | FSK-2 transmission with 2 repetitions: 149.18 Sequence on top of OOK/FSK with 2 repetitions: 151 | Resourceoverhead for 28kbps data rate, which corresponds to the following waveforms to match comparable Msg3 MIL: FSK-2 transmission with 2 repetitions Sequence on top of OOK/FSK with 2 repetitions |
| [8A-5] | LP-WUS+LP-SS | 24 | 1.7770% | 48 symbols for 4 repetitions | 5MHz | 30KHz | 50 paging per second | 50 | 100M, 4 beams |  | OOK transmission with 4 repetitions: 148.68 | Resource overhead for 14kbps data rate, which corresponds to the following waveforms to match comparable Msg3 MIL: OOK transmission with 4 repetitions |
| [8A-16] | LP-WUS | 6 | 0.1692% | 0.6ms | 5MHz | 30KHz | 20 Groups I-DRX | 11.28 | 20MHz | Msg.3 | 147.44 | I-DRX cycle = 1.28sec |
| [8A-16] | LP-WUS | 6 | 0.1692% | 0.6ms | 5MHz | 30KHz | 20 Groups, I-DRX | 11.28 | 20MHz | Msg3 | 147.44 | I-DRX cycle = 1.28sec |
| [8A-16] | LP-WUS | 6 | 0.8461% | 0.6ms | 5MHz | 30KHz | 100 Groups, I-DRX | 56.43 | 20MHz | Msg3 | 147.44 | I-DRX cycle = 1.28sec |
| [8A-16] | LP-WUS | 6 | 0.8461% | 0.6ms | 5MHz | 30KHz | 100 Groups, I-DRX | 56.43 | 20MHz | Msg3 | 147.44 | I-DRX cycle = 1.28sec |
| [8A-16] | LP-WUS | 6 | 0.0036% | 0.6ms | 5MHz | 30KHz | 20 Groups, eDRX | 0.2413 | 20MHz | Msg3 | 147.44 | eDRX cycle = 82.9sec |
| [8A-16] | LP-WUS | 6 | 0.0036% | 0.6ms | 5MHz | 30KHz | 20 Groups, | 0.2413 | 20MHz | Msg3 | 147.44 | eDRX cycle = 82.9sec |
| [8A-16] | LP-WUS | 6 | 0.0181% | 0.6ms | 5MHz | 30KHz | 100 Groups, eDRX | 1.2063 | 20MHz | Msg3 | 147.44 | eDRX cycle = 82.9sec |
| [8A-16] | LP-WUS | 6 | 0.0181% | 0.6ms | 5MHz | 30KHz | 100 Groups, eDRX | 1.2063 | 20MHz | Msg3 | 147.44 | eDRX cycle = 82.9sec |
| [8A-4] | LP-WUS | 16 | 0.2165% | 24 OFDM symbols | 5MHz | 30KHz | 10 | 10 | 20MHz,1 beam | Msg3 | 154.68 | LP-WUS waveforms: OOK-4 when 24 OFDM symbols are used |
| [8A-4] | LP-WUS | 16 | 0.0541% | 24 OFDM symbols | 5MHz | 30KHz | 10 | 2.5 | 20MHz,1 beam | Msg3 | 154.68 |  |
| [8A-4] | LP-WUS | 16 | 0.8660% | 24 OFDM symbols | 5MHz | 30KHz | 10 | 10 | 20MHz,4beam | Msg3 | 154.68 |  |
| [8A-4] | LP-WUS | 16 | 0.2165% | 24 OFDM symbols | 5MHz | 30KHz | 10 | 2.5 | 20MHz,4 beam | Msg3 | 154.68 |  |
| [8A-4] | LP-WUS | 16 | 1.7321% | 24 OFDM symbols | 5MHz | 30KHz | 10 | 10 | 20MHz,8 beam | Msg3 | 154.68 |  |
| [8A-4] | LP-WUS | 16 | 0.4330% | 24 OFDM symbols | 5MHz | 30KHz | 10 | 2.5 | 20MHz,8 beam | Msg3 | 154.68 |  |
| [8A-4] | LP-WUS | 16 | 0.0433% | 24 OFDM symbols | 5MHz | 30KHz | 10 | 10 | 100MHz,1 beam | Msg3 | 154.68 |  |
| [8A-4] | LP-WUS | 16 | 0.0108% | 24 OFDM symbols | 5MHz | 30KHz | 10 | 2.5 | 100MHz,1 beam | Msg3 | 154.68 |  |
| [8A-4] | LP-WUS | 16 | 0.1732% | 24 OFDM symbols | 5MHz | 30KHz | 10 | 10 | 100MHz,4beam | Msg3 | 154.68 |  |
| [8A-4] | LP-WUS | 16 | 0.0433% | 24 OFDM symbols | 5MHz | 30KHz | 10 | 2.5 | 100MHz,4 beam | Msg3 | 154.68 |  |
| [8A-4] | LP-WUS | 16 | 0.3464% | 24 OFDM symbols | 5MHz | 30KHz | 10 | 10 | 100MHz,8 beam | Msg3 | 154.68 |  |
| [8A-4] | LP-WUS | 16 | 0.0866% | 24 OFDM symbols | 5MHz | 30KHz | 10 | 2.5 | 100MHz,8 beam | Msg3 | 154.68 |  |
| [8A-4] | LP-WUS | 16 | 0.0180% | 24 OFDM symbols | 5MHz | 30KHz | 0.8 | 0.8 | 20MHz,1 beam | Msg3 | 154.68 |  |
| [8A-4] | LP-WUS | 16 | 0.0045% | 24 OFDM symbols | 5MHz | 30KHz | 0.8 | 0.2 | 20MHz,1 beam | Msg3 | 154.68 |  |
| [8A-4] | LP-WUS | 16 | 0.0722% | 24 OFDM symbols | 5MHz | 30KHz | 0.8 | 0.8 | 20MHz,4beam | Msg3 | 154.68 |  |
| [8A-4] | LP-WUS | 16 | 0.0180% | 24 OFDM symbols | 5MHz | 30KHz | 0.8 | 0.2 | 20MHz,4 beam | Msg3 | 154.68 |  |
| [8A-4] | LP-WUS | 16 | 0.1443% | 24 OFDM symbols | 5MHz | 30KHz | 0.8 | 0.8 | 20MHz,8 beam | Msg3 | 154.68 |  |
| [8A-4] | LP-WUS | 16 | 0.0361% | 24 OFDM symbols | 5MHz | 30KHz | 0.8 | 0.2 | 20MHz,8 beam | Msg3 | 154.68 |  |
| [8A-4] | LP-WUS | 16 | 0.0036% | 24 OFDM symbols | 5MHz | 30KHz | 0.8 | 0.8 | 100MHz,1 beam | Msg3 | 154.68 |  |
| [8A-4] | LP-WUS | 16 | 0.0009% | 24 OFDM symbols | 5MHz | 30KHz | 0.8 | 0.2 | 100MHz,1 beam | Msg3 | 154.68 |  |
| [8A-4] | LP-WUS | 16 | 0.0144% | 24 OFDM symbols | 5MHz | 30KHz | 0.8 | 0.8 | 100MHz,4beam | Msg3 | 154.68 |  |
| [8A-4] | LP-WUS | 16 | 0.0036% | 24 OFDM symbols | 5MHz | 30KHz | 0.8 | 0.2 | 100MHz,4 beam | Msg3 | 154.68 |  |
| [8A-4] | LP-WUS | 16 | 0.0289% | 24 OFDM symbols | 5MHz | 30KHz | 0.8 | 0.8 | 100MHz,8 beam | Msg3 | 154.68 |  |
| [8A-4] | LP-WUS | 16 | 0.0072% | 24 OFDM symbols | 5MHz | 30KHz | 0.8 | 0.2 | 100MHz,8 beam | Msg3 | 154.68 |  |
| [8A-4] | LP-WUS | 16 | 0.1083% | 12OFDM symbols | 5MHz | 30KHz | 10 | 10 | 20MHz,1 beam | Msg3 | 151.78 | LP-WUS waveform: FSK-2 (M=2) when 12 OFDM symbols are used |
| [8A-4] | LP-WUS | 16 | 0.0271% | 12OFDM symbols | 5MHz | 30KHz | 10 | 2.5 | 20MHz,1 beam | Msg3 | 151.78 |  |
| [8A-4] | LP-WUS | 16 | 0.4330% | 12OFDM symbols | 5MHz | 30KHz | 10 | 10 | 20MHz,4beam | Msg3 | 151.78 |  |
| [8A-4] | LP-WUS | 16 | 0.1083% | 12OFDM symbols | 5MHz | 30KHz | 10 | 2.5 | 20MHz,4 beam | Msg3 | 151.78 |  |
| [8A-4] | LP-WUS | 16 | 0.8660% | 12OFDM symbols | 5MHz | 30KHz | 10 | 10 | 20MHz,8 beam | Msg3 | 151.78 |  |
| [8A-4] | LP-WUS | 16 | 0.2165% | 12OFDM symbols | 5MHz | 30KHz | 10 | 2.5 | 20MHz,8 beam | Msg3 | 151.78 |  |
| [8A-4] | LP-WUS | 16 | 0.0217% | 12OFDM symbols | 5MHz | 30KHz | 10 | 10 | 100MHz,1 beam | Msg3 | 151.78 |  |
| [8A-4] | LP-WUS | 16 | 0.0054% | 12OFDM symbols | 5MHz | 30KHz | 10 | 2.5 | 100MHz,1 beam | Msg3 | 151.78 |  |
| [8A-4] | LP-WUS | 16 | 0.0866% | 12OFDM symbols | 5MHz | 30KHz | 10 | 10 | 100MHz,4beam | Msg3 | 151.78 |  |
| [8A-4] | LP-WUS | 16 | 0.0217% | 12OFDM symbols | 5MHz | 30KHz | 10 | 2.5 | 100MHz,4 beam | Msg3 | 151.78 |  |
| [8A-4] | LP-WUS | 16 | 0.1732% | 12OFDM symbols | 5MHz | 30KHz | 10 | 10 | 100MHz,8 beam | Msg3 | 151.78 |  |
| [8A-4] | LP-WUS | 16 | 0.0433% | 12OFDM symbols | 5MHz | 30KHz | 10 | 2.5 | 100MHz,8 beam | Msg3 | 151.78 |  |
| [8A-4] | LP-WUS | 16 | 0.0090% | 12OFDM symbols | 5MHz | 30KHz | 0.8 | 0.8 | 20MHz,1 beam | Msg3 | 151.78 |  |
| [8A-4] | LP-WUS | 16 | 0.0023% | 12OFDM symbols | 5MHz | 30KHz | 0.8 | 0.2 | 20MHz,1 beam | Msg3 | 151.78 |  |
| [8A-4] | LP-WUS | 16 | 0.0361% | 12OFDM symbols | 5MHz | 30KHz | 0.8 | 0.8 | 20MHz,4beam | Msg3 | 151.78 |  |
| [8A-4] | LP-WUS | 16 | 0.0090% | 12OFDM symbols | 5MHz | 30KHz | 0.8 | 0.2 | 20MHz,4 beam | Msg3 | 151.78 |  |
| [8A-4] | LP-WUS | 16 | 0.0722% | 12OFDM symbols | 5MHz | 30KHz | 0.8 | 0.8 | 20MHz,8 beam | Msg3 | 151.78 |  |
| [8A-4] | LP-WUS | 16 | 0.0180% | 12OFDM symbols | 5MHz | 30KHz | 0.8 | 0.2 | 20MHz,8 beam | Msg3 | 151.78 |  |
| [8A-4] | LP-WUS | 16 | 0.0018% | 12OFDM symbols | 5MHz | 30KHz | 0.8 | 0.8 | 100MHz,1 beam | Msg3 | 151.78 |  |
| [8A-4] | LP-WUS | 16 | 0.0005% | 12OFDM symbols | 5MHz | 30KHz | 0.8 | 0.2 | 100MHz,1 beam | Msg3 | 151.78 |  |
| [8A-4] | LP-WUS | 16 | 0.0072% | 12OFDM symbols | 5MHz | 30KHz | 0.8 | 0.8 | 100MHz,4beam | Msg3 | 151.78 |  |
| [8A-4] | LP-WUS | 16 | 0.0018% | 12OFDM symbols | 5MHz | 30KHz | 0.8 | 0.2 | 100MHz,4 beam | Msg3 | 151.78 |  |
| [8A-4] | LP-WUS | 16 | 0.0144% | 12OFDM symbols | 5MHz | 30KHz | 0.8 | 0.8 | 100MHz,8 beam | Msg3 | 151.78 |  |
| [8A-4] | LP-WUS | 16 | 0.0036% | 12OFDM symbols | 5MHz | 30KHz | 0.8 | 0.2 | 100MHz,8 beam | Msg3 | 151.78 |  |
| [8A-3] | OOK WUS+LP-SS | 1 | 0,0427% | 5 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | Msg3 0rtx | 149.23 | Assume 250 idle UEs per cell, R_E=0.1%, 1UE/subgroup |
| [8A-3] | OOK WUS+LP-SS | 1 | 0,0519% | 8 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OOK WUS+LP-SS | 1 | 0,0549% | 9 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OOK WUS+LP-SS | 1 | 0,0885% | 20 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | PDCCH 2rx | 156.76 |  |
| [8A-3] | OOK WUS+LP-SS | 8 | 0,0580% | 10 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | Msg3 0rtx | 149.23 |  |
| [8A-3] | OOK WUS+LP-SS | 8 | 0,1129% | 28 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OOK WUS+LP-SS | 8 | 0,1434% | 38 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OOK WUS+LP-SS | 8 | 0,2960% | 88 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | PDCCH 2rx | 156.76 |  |
| [8A-3] | OOK WUS+LP-SS | 48 | 0,1251% | 32 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | Msg3 0rtx | 149.23 |  |
| [8A-3] | OOK WUS+LP-SS | 48 | 0,3845% | 117 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OOK WUS+LP-SS | 48 | 0,4547% | 140 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OOK WUS+LP-SS | 48 | 1,2177% | 390 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | PDCCH 2rx | 156.76 |  |
| [8A-3] | OOK WUS+LP-SS | 1 | 0,3418% | 5 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | Msg3 0rtx | 149.23 |  |
| [8A-3] | OOK WUS+LP-SS | 1 | 0,4150% | 8 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OOK WUS+LP-SS | 1 | 0,4395% | 9 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OOK WUS+LP-SS | 1 | 0,7080% | 20 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | PDCCH 2rx | 156.76 |  |
| [8A-3] | OOK WUS+LP-SS | 8 | 0,4639% | 10 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | Msg3 0rtx | 149.23 |  |
| [8A-3] | OOK WUS+LP-SS | 8 | 0,9033% | 28 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OOK WUS+LP-SS | 8 | 1,1475% | 38 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OOK WUS+LP-SS | 8 | 2,3682% | 88 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | PDCCH 2rx | 156.76 |  |
| [8A-3] | OOK WUS+LP-SS | 48 | 1,0010% | 32 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | Msg3 0rtx | 149.23 |  |
| [8A-3] | OOK WUS+LP-SS | 48 | 3,0762% | 117 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OFDM WUS | 1 | 0,00066% | 4 symbols | 3.81MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | Msg3 0rtx | 149.23 |  |
| [8A-3] | OFDM WUS | 1 | 0,00083% | 5 symbols | 3.81MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OFDM WUS | 1 | 0,00100% | 6 symbols | 3.81MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OFDM WUS | 1 | 0,00183% | 11 symbols | 3.81MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | PDCCH 2rx | 156.76 |  |
| [8A-3] | OFDM WUS | 8 | 0,00199% | 12 symbols | 3.81MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | Msg3 0rtx | 149.23 |  |
| [8A-3] | OFDM WUS | 8 | 0,00199% | 12 symbols | 3.81MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OFDM WUS | 8 | 0,00199% | 12 symbols | 3.81MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OFDM WUS | 8 | 0,00249% | 15 symbols | 3.81MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | PDCCH 2rx | 156.76 |  |
| [8A-3] | OFDM WUS | 1 | 0,00532% | 4 symbols | 3.81MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | Msg3 0rtx | 149.23 |  |
| [8A-3] | OFDM WUS | 1 | 0,00664% | 5 symbols | 3.81MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OFDM WUS | 1 | 0,00797% | 6 symbols | 3.81MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OFDM WUS | 1 | 0,01462% | 11 symbols | 3.81MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | PDCCH 2rx | 156.76 |  |
| [8A-3] | OFDM WUS | 8 | 0,01595% | 12 symbols | 3.81MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | Msg3 0rtx | 149.23 |  |
| [8A-3] | OFDM WUS | 8 | 0,01595% | 12 symbols | 3.81MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OFDM WUS | 8 | 0,01595% | 12 symbols | 3.81MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OFDM WUS | 8 | 0,01993% | 15 symbols | 3.81MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | PDCCH 2rx | 156.76 |  |
| [8A-3] | OOK WUS+LP-SS | 1 | 0,0427% | 5 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | Msg3 0rtx | 149.23 | Assume 250 idle UEs per cell, R_E=0.1%, 4UE/subgroup |
| [8A-3] | OOK WUS+LP-SS | 1 | 0,0519% | 8 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OOK WUS+LP-SS | 1 | 0,0549% | 9 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OOK WUS+LP-SS | 1 | 0,0885% | 20 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | PDCCH 2rx | 156.76 |  |
| [8A-3] | OOK WUS+LP-SS | 8 | 0,0580% | 10 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | Msg3 0rtx | 149.23 |  |
| [8A-3] | OOK WUS+LP-SS | 8 | 0,1129% | 28 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OOK WUS+LP-SS | 8 | 0,1434% | 38 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OOK WUS+LP-SS | 8 | 0,2960% | 88 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | PDCCH 2rx | 156.76 |  |
| [8A-3] | OOK WUS+LP-SS | 1 | 0,3418% | 5 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | Msg3 0rtx | 149.23 |  |
| [8A-3] | OOK WUS+LP-SS | 1 | 0,4150% | 8 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OOK WUS+LP-SS | 1 | 0,4395% | 9 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OOK WUS+LP-SS | 1 | 0,7080% | 20 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | PDCCH 2rx | 156.76 |  |
| [8A-3] | OOK WUS+LP-SS | 8 | 0,4639% | 10 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | Msg3 0rtx | 149.23 |  |
| [8A-3] | OOK WUS+LP-SS | 8 | 0,9033% | 28 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OOK WUS+LP-SS | 8 | 1,1475% | 38 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OOK WUS+LP-SS | 8 | 2,3682% | 88 slots | 5MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | PDCCH 2rx | 156.76 |  |
| [8A-3] | OFDM WUS | 1 | 0,00066% | 4 symbols | 3.81MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | Msg3 0rtx | 149.23 |  |
| [8A-3] | OFDM WUS | 1 | 0,00083% | 5 symbols | 3.81MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OFDM WUS | 1 | 0,00100% | 6 symbols | 3.81MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OFDM WUS | 1 | 0,00183% | 11 symbols | 3.81MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | PDCCH 2rx | 156.76 |  |
| [8A-3] | OFDM WUS | 8 | 0,00199% | 12 symbols | 3.81MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | Msg3 0rtx | 149.23 |  |
| [8A-3] | OFDM WUS | 8 | 0,00199% | 12 symbols | 3.81MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OFDM WUS | 8 | 0,00199% | 12 symbols | 3.81MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OFDM WUS | 8 | 0,00249% | 15 symbols | 3.81MHz | 30KHz | 0.25 | 0.25 | 20MHz, 1beam | PDCCH 2rx | 156.76 |  |
| [8A-3] | OFDM WUS | 1 | 0,00532% | 4 symbols | 3.81MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | Msg3 0rtx | 149.23 |  |
| [8A-3] | OFDM WUS | 1 | 0,00664% | 5 symbols | 3.81MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OFDM WUS | 1 | 0,00797% | 6 symbols | 3.81MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OFDM WUS | 1 | 0,01462% | 11 symbols | 3.81MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | PDCCH 2rx | 156.76 |  |
| [8A-3] | OFDM WUS | 8 | 0,01595% | 12 symbols | 3.81MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | Msg3 0rtx | 149.23 |  |
| [8A-3] | OFDM WUS | 8 | 0,01595% | 12 symbols | 3.81MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OFDM WUS | 8 | 0,01595% | 12 symbols | 3.81MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OFDM WUS | 8 | 0,01993% | 15 symbols | 3.81MHz | 30KHz | 0.25 | 0.25 | 20MHz, 8beam | PDCCH 2rx | 156.76 |  |
| [8A-3] |  |  |  |  |  |  |  |  |  |  |  |  |
| [8A-3] | OOK WUS+LP-SS | 1 | 0,0977% | 5 slots | 5MHz | 30KHz | 2.5 | 2.5 | 20MHz, 1beam | Msg3 0rtx | 149.23 | Assume 250 idle UEs per cell, R_E=1%, 1UE/subgroup |
| [8A-3] | OOK WUS+LP-SS | 1 | 0,1892% | 8 slots | 5MHz | 30KHz | 2.5 | 2.5 | 20MHz, 1beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OOK WUS+LP-SS | 1 | 0,2197% | 9 slots | 5MHz | 30KHz | 2.5 | 2.5 | 20MHz, 1beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OOK WUS+LP-SS | 1 | 0,5554% | 20 slots | 5MHz | 30KHz | 2.5 | 2.5 | 20MHz, 1beam | PDCCH 2rx | 156.76 |  |
| [8A-3] | OOK WUS+LP-SS | 8 | 0,2477% | 10 slots | 5MHz | 30KHz | 2.5 | 2.47 | 20MHz, 1beam | Msg3 0rtx | 149.23 |  |
| [8A-3] | OOK WUS+LP-SS | 8 | 0,7904% | 28 slots | 5MHz | 30KHz | 2.5 | 2.47 | 20MHz, 1beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OOK WUS+LP-SS | 8 | 1,0919% | 38 slots | 5MHz | 30KHz | 2.5 | 2.47 | 20MHz, 1beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OOK WUS+LP-SS | 8 | 2,5995% | 88 slots | 5MHz | 30KHz | 2.5 | 2.47 | 20MHz, 1beam | PDCCH 2rx | 156.76 |  |
| [8A-3] | OOK WUS+LP-SS | 48 | 0,9216% | 32 slots | 5MHz | 30KHz | 2.5 | 2.5 | 20MHz, 1beam | Msg3 0rtx | 149.23 |  |
| [8A-3] | OOK WUS+LP-SS | 48 | 3,5156% | 117 slots | 5MHz | 30KHz | 2.5 | 2.5 | 20MHz, 1beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OOK WUS+LP-SS | 48 | 4,2175% | 140 slots | 5MHz | 30KHz | 2.5 | 2.5 | 20MHz, 1beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OOK WUS+LP-SS | 48 | 11,8469% | 390 slots | 5MHz | 30KHz | 2.5 | 2.5 | 20MHz, 1beam | PDCCH 2rx | 156.76 |  |
| [8A-3] | OOK WUS+LP-SS | 1 | 0,7813% | 5 slots | 5MHz | 30KHz | 2.5 | 2.5 | 20MHz, 8beam | Msg3 0rtx | 149.23 |  |
| [8A-3] | OOK WUS+LP-SS | 1 | 1,5137% | 8 slots | 5MHz | 30KHz | 2.5 | 2.5 | 20MHz, 8beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OOK WUS+LP-SS | 1 | 1,7578% | 9 slots | 5MHz | 30KHz | 2.5 | 2.5 | 20MHz, 8beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OOK WUS+LP-SS | 1 | 4,4434% | 20 slots | 5MHz | 30KHz | 2.5 | 2.5 | 20MHz, 8beam | PDCCH 2rx | 156.76 |  |
| [8A-3] | OOK WUS+LP-SS | 8 | 1,9814% | 10 slots | 5MHz | 30KHz | 2.5 | 2.47 | 20MHz, 8beam | Msg3 0rtx | 149.23 |  |
| [8A-3] | OOK WUS+LP-SS | 8 | 6,3232% | 28 slots | 5MHz | 30KHz | 2.5 | 2.47 | 20MHz, 8beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OOK WUS+LP-SS | 8 | 8,7354% | 38 slots | 5MHz | 30KHz | 2.5 | 2.47 | 20MHz, 8beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OOK WUS+LP-SS | 8 | 20,7959% | 88 slots | 5MHz | 30KHz | 2.5 | 2.47 | 20MHz, 8beam | PDCCH 2rx | 156.76 |  |
| [8A-3] | OFDM WUS | 1 | 0,0066% | 4 symbols | 3.81MHz | 30KHz | 2.5 | 2.5 | 20MHz, 1beam | Msg3 0rtx | 149.23 |  |
| [8A-3] | OFDM WUS | 1 | 0,0083% | 5 symbols | 3.81MHz | 30KHz | 2.5 | 2.5 | 20MHz, 1beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OFDM WUS | 1 | 0,0100% | 6 symbols | 3.81MHz | 30KHz | 2.5 | 2.5 | 20MHz, 1beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OFDM WUS | 1 | 0,0183% | 11 symbols | 3.81MHz | 30KHz | 2.5 | 2.5 | 20MHz, 1beam | PDCCH 2rx | 156.76 |  |
| [8A-3] | OFDM WUS | 8 | 0,0197% | 12 symbols | 3.81MHz | 30KHz | 2.5 | 2.47 | 20MHz, 1beam | Msg3 0rtx | 149.23 |  |
| [8A-3] | OFDM WUS | 8 | 0,0197% | 12 symbols | 3.81MHz | 30KHz | 2.5 | 2.47 | 20MHz, 1beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OFDM WUS | 8 | 0,0197% | 12 symbols | 3.81MHz | 30KHz | 2.5 | 2.47 | 20MHz, 1beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OFDM WUS | 8 | 0,0246% | 15 symbols | 3.81MHz | 30KHz | 2.5 | 2.47 | 20MHz, 1beam | PDCCH 2rx | 156.76 |  |
| [8A-3] | OFDM WUS | 1 | 0,0532% | 4 symbols | 3.81MHz | 30KHz | 2.5 | 2.5 | 20MHz, 8beam | Msg3 0rtx | 149.23 |  |
| [8A-3] | OFDM WUS | 1 | 0,0664% | 5 symbols | 3.81MHz | 30KHz | 2.5 | 2.5 | 20MHz, 8beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OFDM WUS | 1 | 0,0797% | 6 symbols | 3.81MHz | 30KHz | 2.5 | 2.5 | 20MHz, 8beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OFDM WUS | 1 | 0,1462% | 11 symbols | 3.81MHz | 30KHz | 2.5 | 2.5 | 20MHz, 8beam | PDCCH 2rx | 156.76 |  |
| [8A-3] | OFDM WUS | 8 | 0,1575% | 12 symbols | 3.81MHz | 30KHz | 2.5 | 2.47 | 20MHz, 8beam | Msg3 0rtx | 149.23 |  |
| [8A-3] | OFDM WUS | 8 | 0,1575% | 12 symbols | 3.81MHz | 30KHz | 2.5 | 2.47 | 20MHz, 8beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OFDM WUS | 8 | 0,1575% | 12 symbols | 3.81MHz | 30KHz | 2.5 | 2.47 | 20MHz, 8beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OFDM WUS | 8 | 0,1969% | 15 symbols | 3.81MHz | 30KHz | 2.5 | 2.47 | 20MHz, 8beam | PDCCH 2rx | 156.76 |  |
| [8A-3] | OOK WUS+LP-SS | 1 | 0,0972% | 5 slots | 5MHz | 30KHz | 2.5 | 2.48 | 20MHz, 1beam | Msg3 0rtx | 149.23 | Assume 250 idle UEs per cell, R_E=1%, 4UE/subgroup |
| [8A-3] | OOK WUS+LP-SS | 1 | 0,1880% | 8 slots | 5MHz | 30KHz | 2.5 | 2.48 | 20MHz, 1beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OOK WUS+LP-SS | 1 | 0,2183% | 9 slots | 5MHz | 30KHz | 2.5 | 2.48 | 20MHz, 1beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OOK WUS+LP-SS | 1 | 0,5513% | 20 slots | 5MHz | 30KHz | 2.5 | 2.48 | 20MHz, 1beam | PDCCH 2rx | 156.76 |  |
| [8A-3] | OOK WUS+LP-SS | 8 | 0,2246% | 10 slots | 5MHz | 30KHz | 2.5 | 2.2 | 20MHz, 1beam | Msg3 0rtx | 149.23 |  |
| [8A-3] | OOK WUS+LP-SS | 8 | 0,7080% | 28 slots | 5MHz | 30KHz | 2.5 | 2.2 | 20MHz, 1beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OOK WUS+LP-SS | 8 | 0,9766% | 38 slots | 5MHz | 30KHz | 2.5 | 2.2 | 20MHz, 1beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OOK WUS+LP-SS | 8 | 2,3193% | 88 slots | 5MHz | 30KHz | 2.5 | 2.2 | 20MHz, 1beam | PDCCH 2rx | 156.76 |  |
| [8A-3] | OOK WUS+LP-SS | 1 | 0,7773% | 5 slots | 5MHz | 30KHz | 2.5 | 2.48 | 20MHz, 8beam | Msg3 0rtx | 149.23 |  |
| [8A-3] | OOK WUS+LP-SS | 1 | 1,5039% | 8 slots | 5MHz | 30KHz | 2.5 | 2.48 | 20MHz, 8beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OOK WUS+LP-SS | 1 | 1,7461% | 9 slots | 5MHz | 30KHz | 2.5 | 2.48 | 20MHz, 8beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OOK WUS+LP-SS | 1 | 4,4102% | 20 slots | 5MHz | 30KHz | 2.5 | 2.48 | 20MHz, 8beam | PDCCH 2rx | 156.76 |  |
| [8A-3] | OOK WUS+LP-SS | 8 | 1,7969% | 10 slots | 5MHz | 30KHz | 2.5 | 2.2 | 20MHz, 8beam | Msg3 0rtx | 149.23 |  |
| [8A-3] | OOK WUS+LP-SS | 8 | 5,6641% | 28 slots | 5MHz | 30KHz | 2.5 | 2.2 | 20MHz, 8beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OOK WUS+LP-SS | 8 | 7,8125% | 38 slots | 5MHz | 30KHz | 2.5 | 2.2 | 20MHz, 8beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OOK WUS+LP-SS | 8 | 18,5547% | 88 slots | 5MHz | 30KHz | 2.5 | 2.2 | 20MHz, 8beam | PDCCH 2rx | 156.76 |  |
| [8A-3] | OFDM WUS | 1 | 0,0066% | 4 symbols | 3.81MHz | 30KHz | 2.5 | 2.48 | 20MHz, 1beam | Msg3 0rtx | 149.23 |  |
| [8A-3] | OFDM WUS | 1 | 0,0082% | 5 symbols | 3.81MHz | 30KHz | 2.5 | 2.48 | 20MHz, 1beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OFDM WUS | 1 | 0,0099% | 6 symbols | 3.81MHz | 30KHz | 2.5 | 2.48 | 20MHz, 1beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OFDM WUS | 1 | 0,0181% | 11 symbols | 3.81MHz | 30KHz | 2.5 | 2.48 | 20MHz, 1beam | PDCCH 2rx | 156.76 |  |
| [8A-3] | OFDM WUS | 8 | 0,0175% | 12 symbols | 3.81MHz | 30KHz | 2.5 | 2.2 | 20MHz, 1beam | Msg3 0rtx | 149.23 |  |
| [8A-3] | OFDM WUS | 8 | 0,0175% | 12 symbols | 3.81MHz | 30KHz | 2.5 | 2.2 | 20MHz, 1beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OFDM WUS | 8 | 0,0175% | 12 symbols | 3.81MHz | 30KHz | 2.5 | 2.2 | 20MHz, 1beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OFDM WUS | 8 | 0,0219% | 15 symbols | 3.81MHz | 30KHz | 2.5 | 2.2 | 20MHz, 1beam | PDCCH 2rx | 156.76 |  |
| [8A-3] | OFDM WUS | 1 | 0,0527% | 4 symbols | 3.81MHz | 30KHz | 2.5 | 2.48 | 20MHz, 8beam | Msg3 0rtx | 149.23 |  |
| [8A-3] | OFDM WUS | 1 | 0,0659% | 5 symbols | 3.81MHz | 30KHz | 2.5 | 2.48 | 20MHz, 8beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OFDM WUS | 1 | 0,0791% | 6 symbols | 3.81MHz | 30KHz | 2.5 | 2.48 | 20MHz, 8beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OFDM WUS | 1 | 0,1450% | 11 symbols | 3.81MHz | 30KHz | 2.5 | 2.48 | 20MHz, 8beam | PDCCH 2rx | 156.76 |  |
| [8A-3] | OFDM WUS | 8 | 0,1403% | 12 symbols | 3.81MHz | 30KHz | 2.5 | 2.2 | 20MHz, 8beam | Msg3 0rtx | 149.23 |  |
| [8A-3] | OFDM WUS | 8 | 0,1403% | 12 symbols | 3.81MHz | 30KHz | 2.5 | 2.2 | 20MHz, 8beam | Msg3 2rtx | 153.23 |  |
| [8A-3] | OFDM WUS | 8 | 0,1403% | 12 symbols | 3.81MHz | 30KHz | 2.5 | 2.2 | 20MHz, 8beam | PDCCH 1rx | 153.76 |  |
| [8A-3] | OFDM WUS | 8 | 0,1754% | 15 symbols | 3.81MHz | 30KHz | 2.5 | 2.2 | 20MHz, 8beam | PDCCH 2rx | 156.76 |  |
| [8A-9] | LP-WUS | 48 | 0.0720% | 8 | 5.04 | 30 | 10 | 10 | 20 | Msg3 | 143.78 | 1 beam LP-WUS is assumed. OOK-4 when 8 OFDM symbols are used. OOK-1 when 64 OFDM symbols are used |
| [8A-9] | LP-WUS | 48 | 0.2808% | 8 | 5.04 | 30 | 39 | 39 | 20 | Msg3 | 143.78 |  |
| [8A-9] | LP-WUS | 48 | 0.3600% | 8 | 5.04 | 30 | 50 | 50 | 20 | Msg3 | 143.78 |  |
| [8A-9] | LP-WUS | 48 | 0.5760% | 64 | 5.04 | 30 | 10 | 10 | 20 | Msg3 | 141.49 |  |
| [8A-9] | LP-WUS | 48 | 2.2464% | 64 | 5.04 | 30 | 39 | 39 | 20 | Msg3 | 141.49 |  |
| [8A-9] | LP-WUS | 48 | 2.8800% | 64 | 5.04 | 30 | 50 | 50 | 20 | Msg3 | 141.49 |  |
| [8A-9] | LP-WUS | 48 | 0.0144% | 8 | 5.04 | 30 | 10 | 10 | 100 | Msg3 | 143.78 |  |
| [8A-9] | LP-WUS | 48 | 0.0562% | 8 | 5.04 | 30 | 39 | 39 | 100 | Msg3 | 143.78 |  |
| [8A-9] | LP-WUS | 48 | 0.0720% | 8 | 5.04 | 30 | 50 | 50 | 100 | Msg3 | 143.78 |  |
| [8A-9] | LP-WUS | 48 | 0.1152% | 64 | 5.04 | 30 | 10 | 10 | 100 | Msg3 | 141.49 |  |
| [8A-9] | LP-WUS | 48 | 0.4493% | 64 | 5.04 | 30 | 39 | 39 | 100 | Msg3 | 141.49 |  |
| [8A-9] | LP-WUS | 48 | 0.5760% | 64 | 5.04 | 30 | 50 | 50 | 100 | Msg3 | 141.49 |  |
| [8A-9] | LP-WUS | 48 | 0.0072% | 8 | 5.04 | 30 | 10 | 1 | 20 | Msg3 | 143.78 |  |
| [8A-9] | LP-WUS | 48 | 0.0281% | 8 | 5.04 | 30 | 39 | 3.9 | 20 | Msg3 | 143.78 |  |
| [8A-9] | LP-WUS | 48 | 0.0360% | 8 | 5.04 | 30 | 50 | 5 | 20 | Msg3 | 143.78 |  |
| [8A-9] | LP-WUS | 48 | 0.0576% | 64 | 5.04 | 30 | 10 | 1 | 20 | Msg3 | 141.49 |  |
| [8A-9] | LP-WUS | 48 | 0.2246% | 64 | 5.04 | 30 | 39 | 3.9 | 20 | Msg3 | 141.49 |  |
| [8A-9] | LP-WUS | 48 | 0.2880% | 64 | 5.04 | 30 | 50 | 5 | 20 | Msg3 | 141.49 |  |
| [8A-9] | LP-WUS | 48 | 0.0014% | 8 | 5.04 | 30 | 10 | 1 | 100 | Msg3 | 143.78 |  |
| [8A-9] | LP-WUS | 48 | 0.0056% | 8 | 5.04 | 30 | 39 | 3.9 | 100 | Msg3 | 143.78 |  |
| [8A-9] | LP-WUS | 48 | 0.0072% | 8 | 5.04 | 30 | 50 | 5 | 100 | Msg3 | 143.78 |  |
| [8A-9] | LP-WUS | 48 | 0.0115% | 64 | 5.04 | 30 | 10 | 1 | 100 | Msg3 | 141.49 |  |
| [8A-9] | LP-WUS | 48 | 0.0449% | 64 | 5.04 | 30 | 39 | 3.9 | 100 | Msg3 | 141.49 |  |
| [8A-9] | LP-WUS | 48 | 0.0576% | 64 | 5.04 | 30 | 50 | 5 | 100 | Msg3 | 141.49 |  |

##### 8.5.1.1.2	LP-SS

The System overhead of LP-SS for RRC IDLE/INACTIVE is summarized in Table 8.5.1-2

Table 8.5.1 - 2 Summary of System overhead for LP-SS for RRC IDLE/INACTIVE

| Source | Signal | Overhead (%) | T resources per beam | F resources | SCS | number of pages UE per second or signal periodicity | System configuration, e.g., BW and etc. | the target channel | LP-SS MIL | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [8A-11] | LP-SS | 0.0630% | 14 symbols | 5.04MHz | 30kHz | 320ms | 100MHz, 8beam | > Msg.3 | 150.78 | The LP-SS coverage is better than msg.3 coverage; The derivation of LP-SS frequency: 4.32MHz +2RB (30kHz) = 5.04MHz |
| [8A-11] | LP-SS | 0.0158% | 14 symbols | 5.04MHz | 30kHz | 1280ms | 100MHz, 8beam | > Msg.3 | 150.78 |  |
| [8A-11] | LP-SS | 0.0020% | 14 symbols | 5.04MHz | 30kHz | 10240ms | 100MHz, 8beam | > Msg.3 | 150.78 |  |
| [8A-11] | LP-SS | 0.3150% | 14 symbols | 5.04MHz | 30kHz | 320ms | 20MHz, 8beam | > Msg.3 | 150.78 |  |
| [8A-11] | LP-SS | 0.0790% | 14 symbols | 5.04MHz | 30kHz | 1280ms | 20MHz, 8beam | > Msg.3 | 150.78 |  |
| [8A-11] | LP-SS | 0.0100% | 14 symbols | 5.04MHz | 30kHz | 10240ms | 20MHz, 8beam | > Msg.3 | 150.78 |  |
| [8A-13] | LP-SS | 0.125%, | 14 symbols | 5MHz | 15KHz | 160ms | 100M, 4beam | Msg.3 | 158.44 | 2.6GHz for LP-SS MIL |
| [8A-13] | LP-SS | 0.625%, | 14 symbols | 5MHz | 15KHz | 160ms | 20M, 4beam | Msg.3 | 158.44 |  |
| [8A-13] | LP-SS | 0.0625%, | 14 symbols | 5MHz | 15KHz | 320ms | 100M, 4beam | Msg.3 | 158.44 |  |
| [8A-13] | LP-SS | 0.3125% | 14 symbols | 5MHz | 15KHz | 320ms | 20M, 4beam | Msg.3 | 158.44 |  |
| [8A-13] | LP-SS | 0.0625%, | 14 symbols | 5MHz | 30KHz | 160ms | 100M, 4beam | Msg.3 | 158.44 |  |
| [8A-13] | LP-SS | 0.3125% | 14 symbols | 5MHz | 30KHz | 160ms | 20M, 4beam | Msg.3 | 158.44 |  |
| [8A-13] | LP-SS | 0.125%, | 14 symbols | 5MHz | 30KHz | 160ms | 100M, 8beam | Msg.3 | 158.44 |  |
| [8A-13] | LP-SS | 0.6250% | 14 symbols | 5MHz | 30KHz | 160ms | 20M, 8beam | Msg.3 | 158.44 |  |
| [8A-13] | LP-SS | 0.03125%, | 14 symbols | 5MHz | 30KHz | 320ms | 100M, 4beam | Msg.3 | 158.44 |  |
| [8A-13] | LP-SS | 0.1563% | 14 symbols | 5MHz | 30KHz | 320ms | 20M, 4beam | Msg.3 | 158.44 |  |
| [8A-13] | LP-SS | 0.0625%, | 14 symbols | 5MHz | 30KHz | 320ms | 100M, 8beam | Msg.3 | 158.44 |  |
| [8A-13] | LP-SS | 0.3125% | 14 symbols | 5MHz | 30KHz | 320ms | 20M, 8beam | Msg.3 | 158.44 |  |
| [8A-5] | LP-SS | 0.0160% | 28symbols | 5MHz | 30KHz | 320ms | 100M, 1beam |  |  |  |
| [8A-5] | LP-SS | 0.0630% | 28symbols | 5MHz | 30KHz | 320ms | 100M, 4beam |  |  |  |
| [8A-16] | LP-SS | 0.1000% | 1ms, 4 repetitions | 5MHz | 30KHz | 1000ms | 20MHz | Msg.3 | 147.44 |  |
| [8A-16] | LP-SS | 0.2000% | 1ms, 4 repetitions | 5MHz | 30KHz | 500ms | 20MHz | Msg.3 | 147.44 |  |
| [8A-16] | LP-SS | 0.5000% | 1ms, 4 repetitions | 5MHz | 30KHz | 200ms | 20MHz | Msg.3 | 147.44 |  |
| [8A-16] | LP-SS | 0.1500% | 1.5ms, 4 repetitions | 5MHz | 30KHz | 1000ms | 20MHz | Msg.3 | 147.44 |  |
| [8A-16] | LP-SS | 0.3000% | 1.5ms, 4 repetitions | 5MHz | 30KHz | 500ms | 20MHz | Msg.3 | 147.44 |  |
| [8A-16] | LP-SS | 0.7500% | 1.5ms, 4 repetitions | 5MHz | 30KHz | 200ms | 20MHz | Msg.3 | 147.44 |  |
| [8A-4] | LP-SS | 0.0113% | 4 OFDM symbols | 5MHz | 30KHz | 320 ms | 20MHz, 1beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0056% | 4 OFDM symbols | 5MHz | 30KHz | 640 ms | 20MHz, 1beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0028% | 4 OFDM symbols | 5MHz | 30KHz | 1280 ms | 20MHz, 1beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0014% | 4 OFDM symbols | 5MHz | 30KHz | 2560 ms | 20MHz, 1beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0007% | 4 OFDM symbols | 5MHz | 30KHz | 5120 ms | 20MHz, 1beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0004% | 4 OFDM symbols | 5MHz | 30KHz | 10240 ms | 20MHz, 1beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0450% | 4 OFDM symbols | 5MHz | 30KHz | 320 ms | 20MHz, 4beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0225% | 4 OFDM symbols | 5MHz | 30KHz | 640 ms | 20MHz, 4beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0113% | 4 OFDM symbols | 5MHz | 30KHz | 1280 ms | 20MHz, 4beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0056% | 4 OFDM symbols | 5MHz | 30KHz | 2560 ms | 20MHz, 4beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0028% | 4 OFDM symbols | 5MHz | 30KHz | 5120 ms | 20MHz, 4beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0014% | 4 OFDM symbols | 5MHz | 30KHz | 10240 ms | 20MHz, 4beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0900% | 4 OFDM symbols | 5MHz | 30KHz | 320 ms | 20MHz, 8beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0450% | 4 OFDM symbols | 5MHz | 30KHz | 640 ms | 20MHz, 8beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0225% | 4 OFDM symbols | 5MHz | 30KHz | 1280 ms | 20MHz, 8beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0113% | 4 OFDM symbols | 5MHz | 30KHz | 2560 ms | 20MHz, 8beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0056% | 4 OFDM symbols | 5MHz | 30KHz | 5120 ms | 20MHz, 8beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0028% | 4 OFDM symbols | 5MHz | 30KHz | 10240 ms | 20MHz, 8beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0023% | 4 OFDM symbols | 5MHz | 30KHz | 320 ms | 100MHz, 1beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0011% | 4 OFDM symbols | 5MHz | 30KHz | 640 ms | 100MHz, 1beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0006% | 4 OFDM symbols | 5MHz | 30KHz | 1280 ms | 100MHz, 1beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0003% | 4 OFDM symbols | 5MHz | 30KHz | 2560 ms | 100MHz, 1beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0001% | 4 OFDM symbols | 5MHz | 30KHz | 5120 ms | 100MHz, 1beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0001% | 4 OFDM symbols | 5MHz | 30KHz | 10240 ms | 100MHz, 1beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0090% | 4 OFDM symbols | 5MHz | 30KHz | 320 ms | 100MHz, 4beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0045% | 4 OFDM symbols | 5MHz | 30KHz | 640 ms | 100MHz, 4beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0023% | 4 OFDM symbols | 5MHz | 30KHz | 1280 ms | 100MHz, 4beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0011% | 4 OFDM symbols | 5MHz | 30KHz | 2560 ms | 100MHz, 4beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0006% | 4 OFDM symbols | 5MHz | 30KHz | 5120 ms | 100MHz, 4beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0003% | 4 OFDM symbols | 5MHz | 30KHz | 10240 ms | 100MHz, 4beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0180% | 4 OFDM symbols | 5MHz | 30KHz | 320 ms | 100MHz, 8beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0090% | 4 OFDM symbols | 5MHz | 30KHz | 640 ms | 100MHz, 8beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0045% | 4 OFDM symbols | 5MHz | 30KHz | 1280 ms | 100MHz, 8beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0023% | 4 OFDM symbols | 5MHz | 30KHz | 2560 ms | 100MHz, 8beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0011% | 4 OFDM symbols | 5MHz | 30KHz | 5120 ms | 100MHz, 8beams | Msg.3 | 155.18 |  |
| [8A-4] | LP-SS | 0.0006% | 4 OFDM symbols | 5MHz | 30KHz | 10240 ms | 100MHz, 8beams | Msg.3 | 155.18 |  |

#### 8.5.1.2	RRC CONNECTED

The System overhead of LP-WUS for RRC CONNECTED is summarized in Table 8.5.1 - 3

Table 8.5.1 -3 Summary of System overhead for LP-WUS for RRC CONNECTED

| Source | Signal | # of info bits carried by signal | Overhead (%) | T resources per beam | F resources | SCS | traffic types | Number of UEs per cell | System configuration, e.g., BW and etc. | the target channel | LP-WUS MIL | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [8A-11] | LP-WUS | 1 | 0.8600% | 8 symbols | 5.04MHz | 30kHz | XR (16.67ms) | 10 | 100MHz | unicast PDCCH AL2-4Rx | 157.99 | The coverage of LP-WUS with 8/32/40 symbols is comparable to unicast PDCCH AL2-4Rx coverage. |
| [8A-11] | LP-WUS | 8 | 3.4300% | 32 symbols | 5.04MHz | 30kHz | XR (16.67ms) | 10 | 100MHz | unicast PDCCH AL2-4Rx | 158.23 |  |
| [8A-11] | LP-WUS | 12 | 4.2900% | 40 symbols | 5.04MHz | 30kHz | XR (16.67ms) | 10 |  | unicast PDCCH AL2-4Rx | 158.07 |  |
| [8A-11] | LP-WUS | 1 | 0.0700% | 8 symbols | 5.04MHz | 30kHz | FTP3 (200ms) | 10 | 100MHz | unicast PDCCH AL2-4Rx | 157.99 |  |
| [8A-11] | LP-WUS | 8 | 0.2900% | 32 symbols | 5.04MHz | 30kHz | FTP3 (200ms) | 10 | 100MHz | unicast PDCCH AL2-4Rx | 158.23 |  |
| [8A-11] | LP-WUS | 12 | 0.3600% | 40 symbols | 5.04MHz | 30kHz | FTP3 (200ms) | 10 | 100MHz | unicast PDCCH AL2-4Rx | 158.07 |  |

### 8.5.2	Observations

For RRC IDLE/INACTIVE,

- For LP-WUS carrying 1-bit information, depending on the time-frequency occupation, traffic inter-arrival time, number of beams, system BW, the system overhead varies,

  - Results in [8A-11] shows that for OOK based LP-WUS system overhead is [0.003%~0.055%] for LP-WUS with 8 beams, 100MHz system BW based on varies configurations as described in the Table 8.5.1 - 1 (per beam LP-WUS duration is 1 symbol, coverage target is PUSCH Msg.3)

  - Results in [8A-11] shows that for OOK based LP-WUS, system overhead is [0.018%~0.28%] for LP-WUS with 8 beams, 20MHz system BW based on varies configurations as described in the Table 8.5.1 – 1 (LP-WUS duration is 1symbol coverage target is PUSCH Msg.3)

  - Results in [8A-3] shows that for LP-WUS with 1 or 8 beams, 20MHz system BW based on varies configurations as described in the Table 8.5.1 – 1, and coverage target is PUSCH Msg.3 with and without retransmissions (2 retransmissions)

      - For OOK based LP-WUS+LPSS with duration 5 or 8 slots, the system overhead is [0.0427%~1.5137%]

      - For OFDM based LP-WUS with duration 4 or 5 symbols, the system overhead is [0.00066%~0.0664%]

  - Results in [8A-3] shows that for LP-WUS with 8 beams, 20MHz system BW based on varies configurations as described in the Table 8.5.1 – 1, and coverage target is PDCCH AL[16] with 1Rx or 2Rx

      - For OOK based LP-WUS+LPSS with duration 9~20 slots, the system overhead is [0.0549%~4.4434%]

      - For OFDM based LP-WUS with duration 6~11 symbols, the system overhead is [0.001%~0.1462%]

- For LP-WUS carrying multiple bits information, depending on the time-frequency occupation, traffic inter-arrival time, number of beams, system BW, the system overhead varies,

  - For LP-WUS carrying 48 bit information,

      - Results in [8A-3] shows that for LP-WUS with 1 or 8 beams, 20MHz system BW based on varies configurations as described in the Table 8.5.1 – 1

        - For OOK based LP-WUS+LPSS with duration 32 or 117 slots, the system overhead is [0.1251%~3.5156%], assuming coverage target is PUSCH Msg.3 with and without retransmissions (2 retransmissions),

        - For OOK based LP-WUS+LPSS with duration 140 or 390 slots, the system overhead is [0.4547%~11.8469%], assuming coverage target is PDCCH AL[16] with 1Rx or 2Rx

      - Results in [8A-9] shows that system overhead is [0.0014%~0.5760%] for OOK based LP-WUS with 1 beam, 100MHz system BW. (per beam LP-WUS duration is 8/64 symbols, coverage target is PUSCH Msg.3)

      - Results in [8A-9] shows that system overhead is [0.0072%~2.8800%] for OOK based LP-WUS with 1 beam, 20MHz system BW. (per beam LP-WUS duration is 8/64 symbols, coverage target is PUSCH Msg.3)

  - For LP-WUS carrying 6~24 bits information,

    - Results in [8A-3] [8A-4] [8A-11][8A-13][8A-16] show that system overhead is [0.0023%~1.9814%], for 6~24bits LP-WUS with 1~8 beams, 20MHz system BW;(per beam LP-WUS duration is 4/5/7/12/14/16.8/24 symbols or 10 slots, coverage target is PUSCH Msg.3 without retransmission)

          - The evaluated waveform for LP-WUS includes OOK, FSK-2

    - Results in [8A-3] shows that system overhead is [0.0580%~1.9814 %], for 8bits OOK based LP-WUS+LP-SS with 1 or 8 beams, 20MHz system BW;(per beam LP-WUS duration is 10 slots, coverage target is PUSCH Msg.3 without retransmission)

    - Results in [8A-3] shows that system overhead is [0.1129%~6.3232%], for 8bits OOK based LP-WUS+LP-SS with 1 or 8 beams, 20MHz system BW;(per beam LP-WUS duration is 28 slots, coverage target is PUSCH Msg.3 two retransmission)

    - Results in [8A-3] shows that system overhead is [0.1434%~20.7959%], for 8bits OOK based LP-WUS+LP-SS with 1 or 8 beams, 20MHz system BW;(per beam LP-WUS duration is 38 or 88 slots, coverage target is PDCCH [AL16] with 1Rx or 2Rx)

    - Results in [8A-3] shows that system overhead is [0.00199%~0.1575%], for 8bits OFDM based LP-WUS with 1 or 8 beams, 20MHz system BW;(per beam LP-WUS duration is 12symbols coverage target is PUSCH Msg.3 without retransmission or with two transmissions)

    - Results in [8A-3] shows that system overhead is [0.00199%~0.1969%], for 8bits OFDM based LP-WUS with 1 or 8 beams, 20MHz system BW;(per beam LP-WUS duration is 12 or 15 symbols, coverage target is PDCCH [AL16] with 1Rx or 2Rx)

    - Results in [8A-4] [8A-5] [8A-11] shows that system overhead is [0.0005%~1.777%] for 7~24bits LP-WUS with 1~8 beams, 100MHz system BW. (per beam LP-WUS duration is 4/5/12/24 symbols or 12symbols with 1/2/4 repetitions, coverage target is PUSCH Msg.3)

          - The evaluated waveform for LP-WUS includes OOK, FSK-2, sequence on top of OOK/FSK and OFDM sequence

- For LP-SS based on OOK with 320ms * X periodicity, 1 slot (30kHz) *Y and 5MHz * Z time-frequency occupation, with 8 beams*W and 20MHz*T system bandwidth, the system overhead is no more than 0.4% * Y * Z * W / X / T.

- Example setting for LP-SS,

    - Set 1: approximately 5MHz LP-SS BW, 1 slot (30KHz) duration, 320ms periodicity, 8 beams, 20MHz system bandwidth (overhead no more than 0.4%).

    - Set 2: approximately 5MHz LP-SS BW, 1 slot (30KHz) duration, 160ms periodicity, 8 beams, 20MHz system bandwidth (overhead no more than 0.8%).

    - Set 3: approximately 5MHz LP-SS BW, 2 slot (30KHz) duration, 320ms periodicity, 8 beams, 20MHz system bandwidth (overhead no more than 0.8%).

    - Set 4: approximately 5MHz LP-SS BW, 1 slot (30KHz) duration, 1000ms periodicity, 8 beams, 20MHz system bandwidth (overhead no more than 0.1%).

    - Note: the overhead for the above reference sets are derived based on the above formula.

For RRC CONNECTED,

- Results in [8A-11] shows that

For XR (16.67ms, 60FPS) and 10 UEs per cell (maximum system capacity), 100MHz system BW,

    - The overhead of LP-WUS based on OOK with 8 symbol duration and 1-bit information is 0.86%;

    - The overhead of LP-WUS based on OOK with 32/40 symbol duration and 8 or 12-bits information is 3.43%~4.29%;

For FTP3 (200ms mean arrival time) and 10 UEs per cell (RU is 40%), 100MHz system BW,

    - The overhead of LP-WUS based on OOK with 8 symbol duration and 1-bit information is 0.07%;

The overhead of LP-WUS based on OOK with 32/40 symbol duration and 8 or 12-bits information is 0.29%~0.36%;

## 8.6	References

[8A-1]	3GPP R1-2307299	On performance evaluation for low power wake-up signal	APPLE

[8A-2]	3GPP R1-2307062	Remaining issues of Deployment scenarios and evaluation methodologies and preliminary performance results of LP-WUR	CATT

[8A-3]	3GPP R1-2308438	Low power WUS evaluations	Ericsson

[8A-4]	3GPP R1-2306428	Evaluation of LP-WUS and Performance Results	FUTUREWEI

[8A-5]	3GPP R1-2306548	Evaluations for LP-WUS	Huawei, HiSilicon

[8A-6]	3GPP R1-2306691	Discussion on evaluation on LP-WUS	InterDigital

[8A-7]	3GPP R1-2308067	Evaluation on low power WUS	MediaTek

[8A-8]	3GPP R1-2307699	Evaluation on LP-WUS/WUR	Samsung

[8A-9]	3GPP R1-2308563	Evaluation for lower power wake-up signal	OPPO

[8A-10]	3GPP R1-2306665	Discussion on evaluation on low power WUS	Spreadtrum

[8A-11]	3GPP R1-2308249	Evaluation results for R18 LP-WUS/WUR	vivo

[8A-12]	3GPP R1-2306921	Evaluation of low-power WUS	Sony

[8A-13]	3GPP R1-2307142	Evaluation on LP-WUS	ZTE, Sanechips

[8A-14]	3GPP R1-2307356	Evaluation on low power WUS	xiaomi

[8A-15]	3GPP R1-2308234	Low power WUS Evaluation Methodology	Nokia, Nokia Shanghai Bell

[8A-16]	3GPP R1-2307948	Evaluation methodology for LP-WUS	Qualcomm

[8A-17]	3GPP R1-2304803	Evaluations on LP WUS	Intel

[8B-1]	3GPP R1-2306429	LP-WUS Physical Signal Design and Performance	FUTUREWEI

[8B-2]	3GPP R1-2306550	Further details on signal design and procedure for LP-WUS	Huawei, HiSilicon

[8B-3]	3GPP R1-2306626	Discussion on L1 signal design and procedure for low power WUS	EURECOM

[8B-4]	3GPP R1-2306666	Discussion on L1 signal design and procedure for low power WUS	Spreadtrum Communications

[8B-5]	3GPP R1-2306682	L1 signal design and procedure for low power WUS	TCL

[8B-6]	3GPP R1-2306693	Discussion on L1 signal design and procedure for LP-WUS	InterDigital, Inc.

[8B-7]	3GPP R1-2306771	Discussion on physical signal and procedure for low power WUS	vivo

[8B-8]	3GPP R1-2306805	Discussion on low power wake up signal design	Panasonic

[8B-9]	3GPP R1-2306922	On L1 signal design and procedures for LP-WUS	Sony

[8B-10]	3GPP R1-2306927	WUS Design Considerations and Impact on WUR Power	Everactive

[8B-11]	3GPP R1-2307064	Physical layer signals/procedures and higher layer protocol for Low-Power WUR	CATT

[8B-12]	3GPP R1-2307116	Discussion on L1 signal design and procedure for LP-WUS	NEC

[8B-13]	3GPP R1-2307144	LP-WUS design and related procedure	ZTE, Sanechips

[8B-14]	3GPP R1-2307216	Discussion on L1 signal design for low power WUS	CMCC

[8B-15]	3GPP R1-2308469	On the L1 signal design and procedures for low power wake-up signal	Apple

[8B-16]	3GPP R1-2307357	Discussions on L1 signal design and procedure for low power WUS	xiaomi

[8B-17]	3GPP R1-2307420	L1 signal design and procedures for low power WUS	Nokia, Nokia Shanghai Bell

[8B-18]	3GPP R1-2307491	L1 signal design and procedure for low power WUS	NTT DOCOMO, INC.

[8B-19]	3GPP R1-2307557	Design consideration on lower power wake-up signal and procedure	OPPO

[8B-20]	3GPP R1-2307625	Discussion on signal design and procedure for LP-WUS	China Telecom

[8B-21]	3GPP R1-2307701	Signal design and procedure for LP-WUS	Samsung

[8B-22]	3GPP R1-2307795	Discussion on L1 signal design and procedure for LP-WUS	LG Electronics

[8B-23]	3GPP R1-2307856	Discussion on L1 signal design and procedure for low power WUS	Sharp

[8B-24]	3GPP R1-2307950	L1 signal design and procedures for LP-WUR	Qualcomm Incorporated

[8B-25]	3GPP R1-2307991	L1 signal design and procedure for low power WUS	Ericsson

[8B-26]	3GPP R1-2308022	On LP-WUS signal design	Nordic Semiconductor ASA

[8B-27]	3GPP R1-2308069	L1 signal design and procedure for low power WUS	MediaTek Inc.

[8B-28]	3GPP R1-2308102	Discussion on the L1 signal design and procedure for low power WUS	Lenovo

[8B-29]	3GPP R1-2308647	R18 SI LLS results   			Moderator (Nordic Semiconductor ASA)

# 9		Conclusions

## 9.1	Power/Latency/Coverage/Overhead/Network Energy

RAN1 has identified use cases of LP-WUS/WUR for power sensitive devices including IoT, wearable and eMBB, and for both IDLE/INACTIVE and CONNECTED mode usage. RAN1 has identified the performance metrics and KPIs for the studies of LP-WUS/WUR including UE power consumption, coverage, latency and UPT as well as metrics and KPIs for network impact including the impact to system overhead, system capacity (XR traffic), and network energy consumption.

RAN1 has developed evaluation methodologies for the study of LP-WUS/WUR including power models for LR and MR, traffic characteristics, coverage targets, LR clocks and oscillator assumptions, etc.

RAN1 has carried out system level evaluations for the UE power saving and latency benefit of using LP-WUS/WUR for IDLE/INACTIVE UEs.

- UE power saving gain cannot be observed if the existing Rel-18 MR RRM measurement periodicity for serving and neighbour cells are applied and UE MR enters in ultra-deep sleep during LP LP-WUS monitoring, therefore MR serving and neighbour cell measurement with further time domain relaxation than that is allowed in Rel-18 specification for IDLE/INACTIVE and/or at least serving cell RRM offloaded from MR to LR are beneficial.

- Compared with existing I-DRX operation,

  - Significant UE power saving gain (up to more than 90%) is observed by using LP-WUS/WUR to trigger UE MR paging monitoring compared with existing I-DRX operation (with and without PEI), if sufficient relaxation to MR RRM measurement is applied.

    - For duty-cycled LP-WUS monitoring and same duty ratio, higher power saving gain is observed if the relative power consumption of LP-WUR ON for LP-WUS monitoring is lower.

    - For continuous LP-WUS monitoring, UE power saving gain is only observed if the relative power consumption of LP-WUR ON for LP-WUS monitoring is lower, e.g. no larger than 1 unit.

  - If the MR enters ultra deep sleep while monitoring LP-WUS, compared with legacy I-DRX operation with same I-DRX cycle, moderate paging latency increase (e.g. ~400ms) is observed by using LP-WUS/WUR, due to the ramp up and re-sync procedure of MR from ultra-deep sleep state and if legacy paging occasion is reused. From RAN1 perspective, potential techniques to decrease the latency e.g. using shorter I-DRX cycles, dynamic paging occasion determination, UE MR transmit PRACH directly after wake-up by LP-WUS, UE MR entering deep sleep during LP LP-WUS monitoring, were proposed and evaluated.

- Compared with existing eDRX operation, significant paging latency reduction and moderate UE power saving gain is observed, if LP-WUS monitoring and the corresponding paging monitoring after MR wake-up is performed not restricted within existing PTW of eDRX. Significant UE power gain and moderate paging latency increase is observed if LP-WUS monitoring is restricted within existing PTW of eDRX and existing paging occasion determination is reused.

RAN1 has carried out system level evaluations for the UE power saving and UPT benefit of using LP-WUS/WUR for CONNETED mode UEs with different traffic types including XR, FTP and IM. Existing power saving techniques, including Rel-15 long/short C-DRX cycle, Rel-16 DCP, Rel-17 PDCCH skipping/SSSG switching and Rel-18 enhanced C-DRX for non-integer periodicities are used for in the evaluation comparison.

- For XR traffic and LP-WUS to trigger the UE MR PDCCH monitoring,

  - Moderate UE power saving gain (up to more than 10%) across different types of XR traffic and system load scenarios can be observed, larger UE power saving gain can be observed if the UE MR can enter light sleep instead of micro sleep during LP-WUS monitoring by LR.

  - The impact to the XR capacity is marginal for low load scenarios. For high load scenarios, the impact to the XR capacity is also marginal when the UE MR enters the micro sleep state and relatively larger when MR enters the light sleep state during LR LP-WUS monitoring.

- For FTP and IM traffic,

  - For the usage of LP-WUS to trigger the UE MR PDCCH monitoring,

    - Compared with existing UE power saving techniques, significant UE power saving gain (up to more than 60%) and moderate UPT improvement (up to more than 10%) when the UE MR enters deep sleep state during LR LP-WUS monitoring, larger UPT improvement (up to more than 180%) is observed for traffic with small packets, e.g. IM. Relatively lower or no UE power saving gain can be observed when the UE MR enters light sleep during LR LP-WUS monitoring, but the UPT improvement is higher (up to more than 50%).

    - Compared with UE MR always-on PDCCH monitoring, significant UE power saving gain (up to more than 40%) and similar UPT performance can be observed, when UE MR enters micro sleep during LR LP-WUS monitoring.

  - For the usage of LP-WUS as the wake-up mechanism of C-DRX similar to DCP

    - Significant UE power saving gain (up to more than 30%) and moderate degradation to the UPT (less than 10%) can be observed, especially when the C-DRX cycle is shorter (e.g. 40ms) .

RAN1 has carried out coverage evaluation to compare the MIL performance of LP-WUS with that of legacy NR channels (e.g. PUSCH for MSG3, PDCCH for Paging). It is observed that for LP-WUS can achieve comparable MIL performance with that of NR PUSCH MSG3, with the assumed resource for each LP-WUS transmission is as the following, in which RedCap and non-RedCap UE cases are not further distinguished.

- For Urban scenario and single PUSCH MSG3 transmission

  - For OOK-based LP-WUS, the required resource reported is 0.9~17.28 MHz*Symbol/bit

  - For FSK-based LP-WUS, the required resource reported is 4.32~25.92 MHz*Symbol/bit

  - For OFDM-based LP-WUS, the required resource reported is 0.31~4.32 MHz*Symbol/bit

- For Urban scenario and PUSCH MSG3 transmission with two retransmissions (one source)

  - For OOK-based LP-WUS, the required resource reported is 241.92 MHz*Symbol/bit

  - For OFDM-based LP-WUS, the required resource reported is 2.16 MHz*Symbol/bit

- For Rural scenarios and single PUSCH MSG3 transmission

  - For OOK-based LP-WUS, the required resource reported is 0.72~4.32 MHz*Symbol/bit

  - For FSK-based LP-WUS, the required resource reported is 4.32 MHz*Symbol/bit (one source)

  - For OFDM-based LP-WUS, the required resource reported is 0.62~4.36 MHz*Symbol/bit

- Note the above OFDM symbol assumes subcarrier spacing 30kHz

- In general, it is much more challenging for LP-WUS to reach comparable MIL as legacy PDCCH with AL16/AL8, more resources occupancy and/or coverage enhancement techniques for LP-WUS transmission would be required to reach such challenging MIL target.

For the overhead of LP-WUS used for RRC IDLE/INACTIVE UEs, it depends on the number of information bits, time-frequency occupation, traffic inter-arrival time, number of beams, system BW.

- For 5MHz LP-WUS with single PUSCH MSG 3 as MIL target

  - For OFDM based LP-WUS carrying information of up to 24bits, the overhead is marginal (up to 0.16%) for 20MHz or 100MHz system BW

  - For OOK/FSK-2 based LP-WUS carrying information up to 24bit, the overhead is marginal (up to 1.98%) for 20MHz. Note that in this case, the overhead evaluation in some sources includes LP-WUS and LP-SS. The reported maximum payload and maximum overhead are from different sources and are not corresponds to each other.

- For 5MHz LP-WUS with paging PDCCH (AL8/AL16) as the MIL target and 20MHz system BW,

  - For OFDM based LP-WUS carrying information up to 8bits, the overhead is marginal (up to 0.19%) (one source)

  - For OOK-based LP-WUS carrying information up to 48bits, the overhead can be significant (up to 21%). Note that in this case, the overhead evaluation in some sources includes LP-WUS and LP-SS

For the overhead of LP-WUS used RRC CONNECTED mode (One source)

- For XR and 10 UEs per cell (maximum system capacity) and 100MHz system BW, the overhead of OOK-based LP-WUS carrying up to 12bits information bits is small (up to 4.3%);

- For FTP3 traffic and 10 UEs per cell, RU 40%, 100MHz system BW, the overhead of OOK-based LP-WUS carrying up to 12 information bits is marginal (up to 0.36%);

For OOK based LP-SS with 5MHz, 8 beams and 20MHz system bandwidth, the overhead is 0.4% for 320ms LP-SS periodicity and 1 slot (30kHz) duration per beam, and 0.3% for 1280ms LP-SS periodicity and 3 slots (30KHz) duration per beam. The overhead of other configurations can be derived by scaling accordingly.

The additional increased network power consumption due to LP-SS is also studied assuming LP-SS is an additional signal transmission than the existing NR signal/channels. When 320ms LP-SS periodicity, 4 or 8 beams and no more than 14 symbols LP-SS duration is assumed, the additional increased network power consumption rate is marginal (0.06%~3.9%), (0.07%~2.716%), (0.388%~1.076%) for zero load, low load and medium load respectively. For 320ms LP-SS periodicity, 8 beams and 42 symbols LP-SS duration, one source shows the additional increase network power consumption rate is 11.4%.

Lower impact to the network power consumption is expected when LP-SS is transmitted FDM with NR SSB/SIB-1.

## 9.2	LP-WUR architecture

For the LP-WUR architecture,

- For OOK waveform, 3 types of receiver architectures have been considered: RF envelope detection, heterodyne architecture with IF envelope detection, and homodyne/zero-IF architecture with baseband envelope detection.

  - For RF envelope detection, the reported relative power consumption for ON state is in the range of 0.01~0.2, and the reported noise figure is in the range of 12~22 dB.

  - For heterodyne architecture, the reported relative power consumption for ON state is in the range of 0.1~4, and the reported noise figure is in the range of 9~15 dB.

  - For homodyne/zero-IF architecture, the reported relative power consumption for ON state is in the range of 0.05~4, and the reported noise figure is in the range of 10~16 dB.

- For FSK waveform, the architectures with parallel OOK receivers and with frequency to amplitude conversion have been considered. Among the architectures that have more than 2 sources providing the analysis,

  - For parallel heterodyne architecture, the reported relative power consumption for ON state is in the range of 0.1~1, and the reported noise figure is in the range of 9~15 dB.

  - For parallel homodyne/zero-IF architecture, the reported relative power consumption for ON state is in the range of 0.1~1, and the reported noise figure is in the range of 10~16 dB.

- For OFDMA-based signals, sequence-based signals have been considered, where the LP WUR performs either time-domain correlation without FFT or frequency-domain correlation after FFT.

  - For time-domain correlation, the reported relative power consumption for ON state is in the range of 0.15~10/30, and the reported noise figure is in the range of 7~25.

  - For frequency-domain correlation, the reported relative power consumption for ON state is in the range of 1~30, and the reported noise figure is in the range of 7~12.

- Note that:

  - Some of the inconsistent ranges for the architectures for OOK and FSK waveforms (e.g., power consumption for the homodyne/zero-IF architecture for OOK and the parallel homodyne/zero-IF architecture for FSK) is due to the fact that not all sources provided analysis for all the architectures.

    - For each individual source, the power consumption for FSK is similar as or slightly higher than the power consumption for OOK with the same architecture type.

  - Note that some of the wide ranges for the different architectures is due to the fact that different sources made different assumptions and there is a tradeoff between power consumption and noise figure.

## 9.3	LLS performance and LP-WUS design/L1 procedure

RAN1 studied robustness of OOK-1, OOK-2, OOK-3, OOK-4, FSK-1, FSK-2 and OFDMA waveform to time and frequency offset in Sections 8.3.1, and 8.3.2 resp. and observed the following:

- OOK/FSK waveform with longer time segment and with a single or multiple frequency segments is comparable (0 us) or more robust (by 4us) to timing error than waveform with shorter time segment.

- One source shows that without sliding window, OFDMA tolerates up to 4us timing error, when SCS <=30kHz, and receiver did not perform FFT.

- For OFDMA, tolerance to timing error varies with sliding window size assumed by a receiver and was shown to tolerate timing error up to 4us if proper sliding window size is assumed by a receiver.

- Single frequency segment OOK (except OOK3) waveform is more robust to frequency error (of 390kHz) than OOK/FSK waveforms with multiple frequency segments (depending on guard-band size between segments) and both are more robust than OFDMA waveform assuming no frequency compensation/synchronization.

- One source showed that single frequency segment FSK-envelop-IF waveform is more robust to frequency error (of 260 kHz) than OOK/FSK waveforms with multiple frequency segments (depending on guard-band size between segments) and both are more robust than OFDMA waveform assuming no frequency compensation/ synchronization.

In RAN1 also the impact of sampling rate on performance has been studied.

RAN1 studied spectral efficiency of OOK-1, OOK-2, OOK-3, OOK-4, FSK-1, FSK-2 and OFDMA waveform, where the best results (of spectral efficiency) from results reported by each company for each waveform are summarized in Section 8.3.3. Results among companies were combined across different receiver types, different power pooling assumption, different sampling rates, different tx antenna configurations, FAR target for the same waveform.

RAN1 studied RSRP and RSRQ measurement accuracy based on LP-SS (based on OOK which can be received by envelop detector) assuming TDL-C channel and observed that depending on SNR target X= {-3, -6, -9, -11} dB as seen by LP-WUR, and depending on 90% accuracy of 3 or 5 dB, different number symbols (1 -70) spread over 1-5 periods is required. Timing and frequency impairments were also considered. RAN1 studied RSRP and RSRQ measurement accuracy based on LP-SS (based on OOK which can be received by envelop detector) assuming AWGN channel and observed that depending on SNR target X= {-9, -11} dB as seen by LP-WUR, and depending on 90% accuracy of 3 or 5 dB, different number symbols (1 -20) spread over 1-3 periods is required. Timing and frequency impairments were also considered. Corresponding SNR observed by MR and LR is different due to NF difference between them. Accuracy of RSRP and RSRQ measurement depends on sampling rate.

RAN1 studied RSRP measurement accuracy based on SSS (OFDMA received by I/Q detector) and observed that depending on SNR target X= [-3, -6] dB as seen by LP-WUR, and depending on 90% accuracy of 3dB, 1 OFDMA symbol in 1 period is required, assuming TDL-C. Timing and frequency impairments were also considered.

For waveform generation of OOK/FSK the following observations were made. Flat spectrum in frequency domain provides robustness against frequency selective fading compared to concentrated energy in frequency domain. For OOK-4, sequence before DFT/LS with variation in phase via such as ZC, M-sequence or QAM sequence can achieve more flattened spectrum. Sequence(s) used in LP-WUS symbol generation with different pulse shape or spectral shape may have different performances. Knowledge of sequence(s) used in LP-WUS waveform generation may improve performance for at least a receiver with I/Q branches.

Pre-storing of the generated frequency domain samples to be mapped to LP-WUS sub-carrier segment of iFFT at gNB may reduce complexity of waveform generation at gNB with memory requirement depending on number of possible combinations. The number of combinations is function of number of supported LP-WUS bandwidth sizes, supported values of M for OOK-4, etc. Pre-storing of the generated frequency domain samples may be up to gNB implementation. For OOK4, Manchester coding will reduce the number of combinations for OOK-4 given the same time/frequency resource.

RAN1 studied LP-WUS bandwidth, at least for IDLE/Inactive mode, at least one BW-size smaller or equal to 5MHz is recommended to be supported for FR1. Other BW sizes are not precluded, however if additional BW-size(s) are recommended to be supported, BW-size can be up to 20MHz. LP-WUS bandwidth size (including guard-bands) is assumed to be an integer number of PRBs. From RAN1 perspective, LP-WUS and signals/channels by MR can be at least on the same carrier in the same band. From RAN1 perspective, for multiplexing with other NR signals and channels, it is beneficial if LP-WUS can be flexibly configured within a carrier.

RAN1 studied synchronization of LP-WUR. At least for LP-WUR that cannot receive existing PSS/SSS, periodic LP-SS signal is beneficial for the following functionalities: (a) RRM measurements by LP-WUR, if supported (b) At least coarse time synchronization of LP-WUR. (c) At least coarse frequency synchronization of LP-WUR. Additional periodic LP-SS system overhead depends on LP-SS periodicity, system BW, number of beams, and resource required to fulfill the target functionality, etc. Periodic signal if used for coarse synchronization may reduce the overhead of signal preceding LP-WUS, if any. LP-SS can be designed to be common among UE groups (cell-specific) and further reduce system overhead. For LP-WUR that can receive existing PSS/SSS potentially assisted by PBCH DMRS/TRS for synchronization, existing PSS/SSS potentially assisted by PBCH DMRS/TRS may be used for above functionality. Periodic LP-SS coverage should be equal or better than that of LP-WUS. For fine time and frequency synchronization, a signal (e.g., preamble) preceding or part of LP-WUS may be used.

OFDMA waveform can provide coverage for LP-WUS with lower resource overhead. LP-WUR receiving OFDMA waveform can reuse PSS/SSS to perform RRM measurement and synchronization avoiding the introduction of periodic LP-SS within the carrier. Timing error robustness can be further improved using a sliding window at the receiver.

Single frequency segment OOK-1/OOK-4 can provide a range of spectral efficiencies while being the most robust waveform to frequency error while robustness to timing error decreases with the increasing of M, but could be addressed by using a sliding window at the receiver or by pulse shaping in time domain. OOK-4 with variable M can provide a flexible range of spectral efficiencies for a fixed LP-WUS resource. Sequences to generate ON duration in OFDMA transmitter, if specified, can help receiver (with I/Q branches) performance.

FSK-2 can provide a range of spectral efficiencies by varying M while having good robustness to frequency error and moderate robustness to timing error. Frequency error robustness can be further improved using frequency error correction (i.e., utilizing 2^M parallel receiver structure or frequency domain sliding window), larger guard band between segments at the expense of less frequency diversity, and/or single frequency segment FSK2-envelope IF. Timing error robustness can be further improved using a sliding window or by pulse shaping in the time domain. Uniform distribution of frequency spectrum density can be achieved using single frequency segment FSK2-envelope IF which can provide robustness against frequency fading. Sequences to generate ON duration in OFDMA transmitter, if specified, can help receiver (with I/Q branches) performance.

# Annex 7.1.1a	Analysis of LP-WUR architectures

RAW data are attached as file in the TR

# Annex 8.1	Power evaluation

RAW data are attached as file in the TR

# Annex 8.2	Coverage

RAW data are attached as file in the TR

# Annex 8.3	LLS results

RAW data can be found in 3GPP R1-2308647.

# Annex 8.4	Network power consumption

## Annex8.4.1	[8A-11]

Table 1. Assumption on baseline and LP-SS configuration

| Parameter | Assumption |
| --- | --- |
| Baseline: SSB and SIB1 transmitted in FDM manner; RACH monitoring | - Periodicity of SSB/SIB1 transmission, RACH monitoring: 20ms - SSB: 4 slots with 2 SSBs in each slot, where 1 SSB occupies 4 OFDM symbols and 20 PRBs - SIB 1: occupies 4 slots and 48 PRBs - RACH: occupies 1 slot |
| System load | Zero, low, light, medium load, which follows the agreed definition in R18 NES SI. |
| LP-SS | - Periodicity of LP-SS: P=320, 640, 1280, 2560, 5120, 10240ms - One LP-SS (per beam) T-F duration is one slot and 11PRBs - 4 or 8 beams assumed for LP-SS |
| FDM pattern | LP-SS fully FDM with SSB/SIB1; No time gap between them |
| TDM pattern | LP-SS is right after SSB/SIB1 |
| Note: the NW power consumption of LP-WUS transmission is not accounted in the simulation due to sparse traffic arrival rate. |  |

Table 2. the Network power consumption caused by LP-SS with 8 beams for RRC idle/inactive mode

| Load | Scheme | Case | Cat 1 |  | Cat 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Mean BS Power consumption | Gain/loss (vs. Baseline) | Mean BS Power consumption | Gain/loss (vs. Baseline) |
| Zero load | FDM with SSB | Baseline | 23.287 |  | 3.466 |  |
|  |  | P=320ms | 23.706 | 1.799% | 3.504 | 1.097% |
|  |  | P=640ms | 23.497 | 0.900% | 3.485 | 0.549% |
|  |  | P=1280ms | 23.392 | 0.450% | 3.476 | 0.274% |
|  |  | P=2560ms | 23.340 | 0.225% | 3.471 | 0.137% |
|  |  | P=5120ms | 23.314 | 0.113% | 3.468 | 0.065% |
|  |  | P=10240ms | 23.302 | 0.061% | 3.467 | 0.031% |
|  | TDM with SSB | Baseline | 23.287 |  | 3.466 |  |
|  |  | P=320ms | 24.114 | 3.551% | 3.538 | 2.076% |
|  |  | P=640ms | 23.701 | 1.776% | 3.502 | 1.038% |
|  |  | P=1280ms | 23.494 | 0.888% | 3.484 | 0.519% |
|  |  | P=2560ms | 23.391 | 0.444% | 3.475 | 0.259% |
|  |  | P=5120ms | 23.332 | 0.192% | 3.470 | 0.124% |
|  |  | P=10240ms | 23.309 | 0.093% | 3.468 | 0.055% |
| Low load | FDM with SSB | Baseline | 27.992 |  | 3.948 |  |
|  |  | P=320ms | 28.364 | 1.328% | 3.984 | 0.912% |
|  |  | P=640ms | 28.168 | 0.627% | 3.966 | 0.459% |
|  |  | P=1280ms | 28.087 | 0.340% | 3.957 | 0.233% |
|  |  | P=2560ms | 28.040 | 0.170% | 3.952 | 0.118% |
|  |  | P=5120ms | 28.017 | 0.087% | 3.950 | 0.057% |
|  |  | P=10240ms | 28.004 | 0.043% | 3.949 | 0.029% |
|  | TDM with SSB | Baseline | 27.992 |  | 3.948 |  |
|  |  | P=320ms | 28.753 | 2.716% | 4.016 | 1.725% |
|  |  | P=640ms | 28.368 | 1.343% | 3.982 | 0.865% |
|  |  | P=1280ms | 28.183 | 0.681% | 3.965 | 0.435% |
|  |  | P=2560ms | 28.090 | 0.350% | 3.956 | 0.222% |
|  |  | P=5120ms | 28.041 | 0.173% | 3.952 | 0.113% |
|  |  | P=10240ms | 28.016 | 0.084% | 3.950 | 0.050% |
| Medium load | FDM with SSB | Baseline | 53.788 |  | 6.649 |  |
|  |  | P=320ms | 54.141 | 0.657% | 6.675 | 0.388% |
|  |  | P=640ms | 53.952 | 0.305% | 6.662 | 0.198% |
|  |  | P=1280ms | 53.875 | 0.162% | 6.656 | 0.105% |
|  |  | P=2560ms | 53.822 | 0.064% | 6.653 | 0.059% |
|  |  | P=5120ms | 53.805 | 0.031% | 6.651 | 0.030% |
|  |  | P=10240ms | 53.796 | 0.015% | 6.650 | 0.015% |
|  | TDM with SSB | Baseline | 53.788 |  | 6.649 |  |
|  |  | P=320ms | 54.367 | 1.076% | 6.698 | 0.742% |
|  |  | P=640ms | 54.137 | 0.648% | 6.674 | 0.375% |
|  |  | P=1280ms | 53.967 | 0.333% | 6.662 | 0.200% |
|  |  | P=2560ms | 53.880 | 0.172% | 6.656 | 0.109% |
|  |  | P=5120ms | 53.831 | 0.080% | 6.653 | 0.053% |
|  |  | P=10240ms | 53.809 | 0.039% | 6.651 | 0.029% |

Table 3. the Network power consumption caused by LP-SS with 4 beams for RRC idle/inactive mode

| Load | Scheme | Case | Cat 1 |  | Cat 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Mean BS Power consumption | Gain/loss (vs. Baseline) | Mean BS Power consumption | Gain/loss (vs. Baseline) |
| Zero load | FDM with SSB | Baseline | 23.2873 |  | 3.4662 |  |
|  |  | P=320ms | 23.3048 | 0.075% | 3.46826 | 0.059% |
|  |  | P=640ms | 23.296 | 0.037% | 3.46723 | 0.030% |
|  |  | P=1280ms | 23.2917 | 0.019% | 3.46671 | 0.015% |
|  |  | P=2560ms | 23.2895 | 0.009% | 3.46645 | 0.007% |
|  |  | P=5120ms | 23.2884 | 0.005% | 3.46633 | 0.004% |
|  |  | P=10240ms | 23.2884 | 0.005% | 3.46633 | 0.004% |
|  | TDM with SSB | Baseline | 23.2873 |  | 3.4662 |  |
|  |  | P=320ms | 23.6888 | 1.724% | 3.50218 | 1.038% |
|  |  | P=640ms | 23.488 | 0.862% | 3.48419 | 0.519% |
|  |  | P=1280ms | 23.3877 | 0.431% | 3.47519 | 0.259% |
|  |  | P=2560ms | 23.3375 | 0.216% | 3.47069 | 0.130% |
|  |  | P=5120ms | 23.3124 | 0.108% | 3.46845 | 0.065% |
|  |  | P=10240ms | 23.3124 | 0.108% | 3.46845 | 0.065% |
| Low load | FDM with SSB | Baseline | 27.99219 |  | 3.947543 |  |
|  |  | P=320ms | 28.00813 | 0.057% | 3.949475 | 0.049% |
|  |  | P=640ms | 28.00016 | 0.028% | 3.948511 | 0.025% |
|  |  | P=1280ms | 27.9962 | 0.014% | 3.948034 | 0.012% |
|  |  | P=2560ms | 27.99417 | 0.007% | 3.947789 | 0.006% |
|  |  | P=5120ms | 27.99328 | 0.004% | 3.947671 | 0.003% |
|  |  | P=10240ms | 27.99328 | 0.004% | 3.947671 | 0.003% |
|  | TDM with SSB | Baseline | 27.99219 |  | 3.947543 |  |
|  |  | P=320ms | 28.34794 | 1.271% | 3.981592 | 0.863% |
|  |  | P=640ms | 28.15971 | 0.598% | 3.964701 | 0.435% |
|  |  | P=1280ms | 28.08326 | 0.325% | 3.956271 | 0.221% |
|  |  | P=2560ms | 28.03778 | 0.163% | 3.95196 | 0.112% |
|  |  | P=5120ms | 28.01555 | 0.083% | 3.949791 | 0.057% |
|  |  | P=10240ms | 28.01555 | 0.083% | 3.949791 | 0.057% |
| Medium load | FDM with SSB | Baseline | 53.7879 |  | 6.649008095 |  |
|  |  | P=320ms | 53.79838571 | 0.019% | 6.65037619 | 0.021% |
|  |  | P=640ms | 53.79342857 | 0.010% | 6.649718571 | 0.011% |
|  |  | P=1280ms | 53.79104286 | 0.006% | 6.649399524 | 0.006% |
|  |  | P=2560ms | 53.78975238 | 0.003% | 6.649232381 | 0.003% |
|  |  | P=5120ms | 53.78898095 | 0.002% | 6.649135238 | 0.002% |
|  |  | P=10240ms | 53.78898095 | 0.002% | 6.649135238 | 0.002% |
|  | TDM with SSB | Baseline | 53.7879 |  | 6.649008095 |  |
|  |  | P=320ms | 54.14139048 | 0.657% | 6.67342 | 0.367% |
|  |  | P=640ms | 53.95587143 | 0.312% | 6.661481905 | 0.188% |
|  |  | P=1280ms | 53.87204286 | 0.156% | 6.655593333 | 0.099% |
|  |  | P=2560ms | 53.83029524 | 0.079% | 6.652702857 | 0.056% |
|  |  | P=5120ms | 53.80868571 | 0.039% | 6.651096667 | 0.031% |
|  |  | P=10240ms | 53.80868571 | 0.039% | 6.651096667 | 0.031% |

## Annex8.4.2	[8A-13]

Assumptions on baseline scheme, LP-SS and LP-WUS configuration are shown in Table 1.

Table 1. Assumption on baseline and LP-SS configuration

| Scheme | Assumption |
| --- | --- |
| Baseline: SSB and SIB1 transmitted in FDM manner | - Periodicity of SSB transmission: 20ms - Periodicity of SIB1 transmission: 40ms - SSB: 4 slots with 2 SSBs in each slot, where 1 SSB occupies 4 OFDM symbols and 20 PRBs - SIB1: occupies 8 slots and 12 PRBs |
| LP-SS and LP-WUS | - Periodicity of LP-SS: 320ms; Offset with SSB: 0 and 10ms; Beams: 4 - Periodicity of LP-WUS: 1280ms; Offset with SSB: 0 and 5ms; Beams: 4 - LP-SS: 4 slots with 1 LP-SS in each slot, where 1 LP-SS occupies 7 OFDM symbols and 12 PRBs - LP-WUS: 4 slots with 1 LP-WUS in each slot, where 1 LP-WUS occupies 14 OFDM symbols and 12 PRBs - No LP-WUS assumption under zero-load scenario |
| Assumption for UE specific wake-up | - Paging cycle: 1280ms - Paging rate: 1% |

Simulation results for zero-load and low-load scenarios based on Cat2 NW power model are shown as Table 2.

Table 2. network power consumption and power increase under LP-SS and LP-WUS configuration

| Cell load | Cases | Network power consumption (Cat2) | Network power increase (%) |
| --- | --- | --- | --- |
| Zero-load scenario | Baseline SSB+SIB1 | 7.0249 |  |
|  | Case1: FDM SSB+SIB1 LP-SS 320-0-4 | 7.0292 | 0.0624 |
|  | Case2: TDM SSB+SIB1 LP-SS 320-10-4 | 7.0602 | 0.5027 |
| Low-load scenario | Baseline SSB+SIB1 | 7.3673 |  |
|  | Case1: FDM SSB+SIB1 LP-SS 320-0-4 LP-WUS 1280-0-4 | 7.3726 | 0.0716 |
|  | Case2: TDM SSB+SIB1 LP-SS 320-10-4 LP-WUS 1280-5-4 | 7.4192 | 0.7048 |

## Annex8.4.3	[8A-3]

| Baseline assumption | Zero load scenario, 8SSBs@20ms, SIB1@160ms |
| --- | --- |

| LP-SS (42 symbol duration (3 slots) per beam, 8 beams) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8SSBs@20ms, SIB1@160ms, LP-SS@160ms | 8SSBs@20ms, SIB1@160ms, LP-SS@320ms | 8SSBs@20ms, SIB1@160ms, LP-SS@640ms | 8SSBs@20ms, SIB1@160ms, LP-SS@1280ms | 8SSBs@20ms, SIB1@160ms, LP-SS@2560ms | 8SSBs@20ms, SIB1@160ms, LP-SS@5120ms | 8SSBs@20ms, SIB1@160ms, LP-SS@10240ms |
| Additional power consumption | 22.8% | 11.4% | 5.7% | 2.8% | 1.4% | 0.7% | 0.4% |

| LP-SS (14 symbol duration (1 slot) per beam, 8 beams) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8SSBs@20ms, SIB1@160ms, LP-SS@160ms | 8SSBs@20ms, SIB1@160ms, LP-SS@320ms | 8SSBs@20ms, SIB1@160ms, LP-SS@640ms | 8SSBs@20ms, SIB1@160ms, LP-SS@1280ms | 8SSBs@20ms, SIB1@160ms, LP-SS@2560ms | 8SSBs@20ms, SIB1@160ms, LP-SS@5120ms | 8SSBs@20ms, SIB1@160ms, LP-SS@10240ms |
| Additional power consumption | 7.7% | 3.9% | 1.9% | 1.0% | 0.5% | 0.2% | 0.1% |

| LP-SS (4 symbols per beam, 8 beams) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 SSBs@20ms, SIB1@160ms, LP-SS@160ms | 8 SSBs@20ms, SIB1@160ms, LP-SS@320ms | 8 SSBs@20ms, SIB1@160ms, LP-SS@640ms | 8 SSBs@20ms, SIB1@160ms, LP-SS@1280ms | 8 SSBs@20ms, SIB1@160ms, LP-SS@2560ms | 8 SSBs@20ms, SIB1@160ms, LP-SS@5120ms | 8 SSBs@20ms, SIB1@160ms, LP-SS@10240ms |
| Additional power consumption | 2.2% | 1.1% | 0.6% | 0.3% | 0.1% | 0.1% | 0.0% |

## Annex8.4.4	[8A-5]

Table 1. Assumptions for network energy consumption evaluation

| System bandwidth | 100M |
| --- | --- |
| SCS | 30KHz |
| LP-SS assumption | Periodicity: 320ms - Time domain resources: 1ms (28 OFDM symbols) - Frequency bandwidth: 5MHz |
| Beam sweeping assumption | - {1, 4} beams |

Table 2. network power consumption due to LP-SS

| Number of paging per second | Beam number | Network energy increasing |
| --- | --- | --- |
| 0 (LP-SS only) | 1 | 0.031% |
| 0 (LP-SS only) | 4 | 0.122% |

# Annex 8.5	System Overhead

## Annex 8.5.1	[8A-11]

Table 1 Resource overhead of LP-WUS to the overall system resource for RRC idle/inactive mode.

| Source | # of info. bits | Overhead (%) | T-F resources | Number of paged UE per second | Number of LP-WUS transmissions per second | System configuration, e.g., BW and beam number etc. | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| [8A-11] | 1 | 0.015% | 1 symbol, 5.04MHz (30kHz SCS) | 10 | 10 | 100MHz, 8beams | For IoT, cases, Assuming 1,000,000 user/km2 connection density [ITU M.2412], with packet arrival every 1 message/2 hours/device [ITU M.2412], assuming 500m ISD with 3 sectors. The coverage of LP-WUS with 1/4/5 symbols is comparable to or better than Msg 3 coverage |
|  | 1 | 0.003% |  | 10 | 3 |  |  |
|  | 8 | 0.06% | 4 symbols, 5.04MHz (30kHz SCS) | 10 | 10 |  |  |
|  | 8 | 0.01% |  | 10 | 3 |  |  |
|  | 8 or 12 | 0.075% | 5 symbols, 5.04MHz (30kHz SCS) | 10 | 10 |  |  |
|  | 8 or 12 | 0.013% |  | 10 | 3 |  |  |
|  | 1 | 0.073% | 1 symbol, 5.04MHz (30kHz SCS) | 10 | 10 | 20MHz, 8beams |  |
|  | 1 | 0.018% |  | 10 | 3 |  |  |
|  | 8 | 0.29% | 4 symbols, 5.04MHz (30kHz SCS) | 10 | 10 |  |  |
|  | 8 | 0.07% |  | 10 | 3 |  |  |
|  | 8 or 12 | 0.363% | 5 symbols, 5.04MHz (30kHz SCS) | 10 | 10 |  |  |
|  | 8 or 12 | 0.088% |  | 10 | 3 |  |  |
|  | 1 | 0.055% | 1 symbol, 5.04MHz (30kHz SCS) | 39 | 39 | 100MHz, 8beams | For eMBB cases, Assuming very high population density, e.g., sports event with 5,000 users/cell population density, and per UE paging arrival rate = 1% when paging i-DRX cycle is Tpagingcycle = 1.28s The coverage of LP-WUS with 1/4/5 symbols is comparable to or better than Msg 3 coverage. |
|  | 1 | 0.015% |  | 39 | 10 |  |  |
|  | 8 | 0.22% | 4 symbols, 5.04MHz (30kHz SCS) | 39 | 39 |  |  |
|  | 8 | 0.06% |  | 39 | 10 |  |  |
|  | 8 or 12 | 0.275% | 5 symbols, 5.04MHz (30kHz SCS) | 39 | 39 |  |  |
|  | 8 or 12 | 0.075% |  | 39 | 10 |  |  |
|  | 1 | 0.280% | 1 symbol, 5.04MHz (30kHz SCS) | 39 | 39 | 20MHz, 8beams |  |
|  | 1 | 0.070% |  | 39 | 10 |  |  |
|  | 8 | 1.12% | 4 symbols, 5.04MHz (30kHz SCS) | 39 | 39 |  |  |
|  | 8 | 0.28% |  | 39 | 10 |  |  |
|  | 8 or 12 | 1.400% | 5 symbols, 5.04MHz (30kHz SCS) | 39 | 39 |  |  |
|  | 8 or 12 | 0.350% |  | 39 | 10 |  |  |

Table 2. Resource overhead of LP-WUS to the overall system resource for RRC connected mode

| Source | # of info. bits | Overhead (%) | T-F resources | Traffic type | Number of UE per cell | System configuration, e.g., BW and beam number etc. | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| [8A-11] | 1 | 0.86% | 8 symbols, 5.04MHz (30kHz SCS) | XR (16.67ms) | 10 (maximum system capacity) | 100MHz, one beam | The coverage of LP-WUS with 8/32/40 symbols is comparable to unicast PDCCH AL2-4Rx coverage. |
|  | 8 | 3.43% | 32 symbols, 5.04MHz (30kHz SCS) |  | 10 |  |  |
|  | 12 | 4.29% | 40 symbols, 5.04MHz (30kHz SCS) |  | 10 |  |  |
|  | 1 | 0.07% | 8 symbols, 5.04MHz (30kHz SCS) | FTP3 (mean packet interval= 200ms) | 10 | 100MHz, one beam |  |
|  | 8 | 0.29% | 32 symbols, 5.04MHz (30kHz SCS) |  | 10 |  |  |
|  | 12 | 0.36% | 40 symbols, 5.04MHz (30kHz SCS) |  | 10 |  |  |

Table 3.  Resource overhead of LP-SS to the system resource for RRC idle/inactive mode

| Source | # of info. bits | Overhead (%) | T-F resources | LP-SS periodicity (ms) | System configuration, e.g., BW and beam number etc. | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| [8A-11] | 1 | 0.063% | 14 symbols, 5.04MHz (30kHz SCS) | 320ms | 100MHz, 8beam | The LP-SS coverage is better than msg.3 coverage; The derivation of LP-SS frequency: 4.32MHz +2RB (30kHz) = 5.04MHz |
|  | 1 | 0.0158% |  | 1280ms | 100MHz, 8beam |  |
|  | 1 | 0.002% |  | 10240ms | 100MHz, 8beam |  |
|  | 1 | 0.315% |  | 320ms | 20MHz, 8beam |  |
|  | 1 | 0.079% |  | 1280ms | 20MHz, 8beam |  |
|  | 1 | 0.01% |  | 10240ms | 20MHz, 8beam |  |

## Annex 8.5.2	[8A-13]

Table 1. Assumptions for system overhead evaluation

| System bandwidth | 100M,20M |
| --- | --- |
| SCS | 15KHz, 30KHz |
| LP-SS assumption | Periodicity{160ms, 320ms} - Time domain resources:14 symbols - Frequency bandwidth: 5MHz |
| LP-WUS assumption | - Time domain resources:N2 symbols: 7, 14 - Frequency bandwidth 5MHz |
| Assumption for UE specific wake-up | - Number of beams:1,4 for 15KHz and 1,4,8 for 30KHz - RE, REF= 1%, 0.1%, 0.01% or 0.001% and YREF = 1.28 |
| Assumption for group specific wake-up | - Number of UEs in a group: 4, 8 |

Table 2. results summary for system overhead

| Source | Signal | Overhead (%) | T-F resources | Inter-arrival time | System configuration, e.g., BW and etc. | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| [8A-13] | LP-SS | 0.125%, 0.625%, | 14 symbols, 5MHz, SCS=15KHz | 160ms | 100M, 20M, 4beam |  |
|  | LP-SS | 0.0625%, 0.3125% | 14 symbols, 5MHz, SCS=15KHz | 320ms | 100M, 20M, 4beam |  |
|  | LP-SS | 0.0625%, 0.3125% | 14 symbols, 5MHz, SCS=30KHz | 160ms | 100M, 20M, 4beam |  |
|  | LP-SS | 0.125%, 0.625% | 14 symbols, 5MHz, SCS=30KHz | 160ms | 100M, 20M, 8beam |  |
|  | LP-SS | 0.03125%, 0.15625% | 14 symbols, 5MHz, SCS=30KHz | 320ms | 100M, 20M, 4beam |  |
|  | LP-SS | 0.0625%, 0.3125% | 14 symbols, 5MHz, SCS=30KHz | 320ms | 100M, 20M, 8beam |  |
|  | LP-WUS | Almost all the cases has <1% system overhead | SCS=15KHz/30kHz Time domain: 7 / 14 symbols Frequency domain: 5M |  | - System bandwidth: 20M, 100M; - Beams:1,4 - Number of UE in a group:1,4,8 - SCS:15KHz,30KHz - OFDM symbols:7,14 - Paging rate:1%,0.1%,0.01%,0.001% - POs number per PF: 1,2,4 - PFs number per cycle: 1,2,4,8,16 | Only one case when 20M system bandwidth, 4beams, no grouping, 15KHz, 14OFDM symbols, paging rate 1%, maximum POs per PF, maximum PFs per cycle are configured, the system overhead may reach 1.6%. |

Table 3. Some example configuration in extreme conditions and system overhead for LP-WUS

| system bandwidth | SCS | grouping | OFDM symbols | beams | UE number | paging rate | PO per PF | PF per cycle | system overhead (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 0 | 1 | 14 | 4 | 20.48 | 0.01 | 4 | 16 | 1.6 |
| 20 | 0 | 1 | 7 | 4 | 20.48 | 0.01 | 4 | 16 | 0.8 |
| 20 | 0 | 1 | 14 | 4 | 10.24 | 0.01 | 4 | 8 | 0.8 |
| 20 | 0 | 1 | 14 | 4 | 10.24 | 0.01 | 2 | 16 | 0.8 |
| 20 | 1 | 1 | 14 | 4 | 20.48 | 0.01 | 4 | 16 | 0.8 |
| 20 | 0 | 1 | 7 | 4 | 10.24 | 0.01 | 4 | 8 | 0.4 |
| 20 | 0 | 1 | 7 | 4 | 10.24 | 0.01 | 2 | 16 | 0.4 |
| 20 | 0 | 1 | 14 | 1 | 20.48 | 0.01 | 4 | 16 | 0.4 |
| 20 | 0 | 1 | 14 | 4 | 5.12 | 0.01 | 4 | 4 | 0.4 |
| 20 | 0 | 1 | 14 | 4 | 5.12 | 0.01 | 2 | 8 | 0.4 |
| 20 | 0 | 1 | 14 | 4 | 5.12 | 0.01 | 1 | 16 | 0.4 |
| 20 | 0 | 4 | 14 | 4 | 20.48 | 0.01 | 4 | 16 | 0.4 |
| 20 | 1 | 1 | 7 | 4 | 20.48 | 0.01 | 4 | 16 | 0.4 |
| 20 | 1 | 1 | 14 | 4 | 10.24 | 0.01 | 4 | 8 | 0.4 |
| 20 | 1 | 1 | 14 | 4 | 10.24 | 0.01 | 2 | 16 | 0.4 |
| 100 | 0 | 1 | 14 | 4 | 20.48 | 0.01 | 4 | 16 | 0.32 |
| 20 | 0 | 1 | 7 | 1 | 20.48 | 0.01 | 4 | 16 | 0.2 |
| 20 | 0 | 1 | 7 | 4 | 5.12 | 0.01 | 4 | 4 | 0.2 |
| 20 | 0 | 1 | 7 | 4 | 5.12 | 0.01 | 2 | 8 | 0.2 |
| 20 | 0 | 1 | 7 | 4 | 5.12 | 0.01 | 1 | 16 | 0.2 |
| 20 | 0 | 1 | 14 | 1 | 10.24 | 0.01 | 4 | 8 | 0.2 |
| 20 | 0 | 1 | 14 | 1 | 10.24 | 0.01 | 2 | 16 | 0.2 |
| 20 | 0 | 1 | 14 | 4 | 2.56 | 0.01 | 4 | 2 | 0.2 |
| 20 | 0 | 1 | 14 | 4 | 2.56 | 0.01 | 2 | 4 | 0.2 |
| 20 | 0 | 1 | 14 | 4 | 2.56 | 0.01 | 1 | 8 | 0.2 |
| 20 | 0 | 4 | 7 | 4 | 20.48 | 0.01 | 4 | 16 | 0.2 |
| 20 | 0 | 4 | 14 | 4 | 10.24 | 0.01 | 4 | 8 | 0.2 |
| 20 | 0 | 4 | 14 | 4 | 10.24 | 0.01 | 2 | 16 | 0.2 |
| 20 | 0 | 8 | 14 | 4 | 20.48 | 0.01 | 4 | 16 | 0.2 |
| 20 | 1 | 1 | 7 | 4 | 10.24 | 0.01 | 4 | 8 | 0.2 |
| 20 | 1 | 1 | 7 | 4 | 10.24 | 0.01 | 2 | 16 | 0.2 |
| 20 | 1 | 1 | 14 | 1 | 20.48 | 0.01 | 4 | 16 | 0.2 |
| 20 | 1 | 1 | 14 | 4 | 5.12 | 0.01 | 4 | 4 | 0.2 |
| 20 | 1 | 1 | 14 | 4 | 5.12 | 0.01 | 2 | 8 | 0.2 |
| 20 | 1 | 1 | 14 | 4 | 5.12 | 0.01 | 1 | 16 | 0.2 |
| 20 | 1 | 4 | 14 | 4 | 20.48 | 0.01 | 4 | 16 | 0.2 |
| 100 | 0 | 1 | 7 | 4 | 20.48 | 0.01 | 4 | 16 | 0.16 |
| 100 | 0 | 1 | 14 | 4 | 10.24 | 0.01 | 4 | 8 | 0.16 |
| 100 | 0 | 1 | 14 | 4 | 10.24 | 0.01 | 2 | 16 | 0.16 |

## Annex 8.5.3	[8A-5]

Table 1. Assumptions for system overhead evaluation

| System bandwidth | 100M |
| --- | --- |
| SCS | 30KHz |
| LP-SS assumption | Periodicity: 320ms - Time domain resources: 1ms (28 OFDM symbols) - Frequency bandwidth: 5MHz |
| LP-WUS assumption | - Bit length: 24 - Time domain resources: {12, 24, 48} OFDM symbols, for {1, 2, 4} repetitions - Frequency bandwidth 5MHz |
| Beam sweeping assumption | - {1, 4} beams |
| Paging number per second | - 0 for LP-SS only (i.e. empty load) - 50 for LP-SS and LP-WUS |

Table 2. results summary for system overhead

| Number of paging per second | Repetition number of LP-WUS | Beam number | Resource overhead |
| --- | --- | --- | --- |
| 0 (LP-SS only) | - | 1 | 0.016% |
| 0 (LP-SS only) | - | 4 | 0.063% |
| 50 (LP-SS+LP-WUS) | 1 | 1 | 0.123% |
| 50 (LP-SS+LP-WUS) | 2 | 1 | 0.230% |
| 50 (LP-SS+LP-WUS) | 4 | 1 | 0.444% |
| 50 (LP-SS+LP-WUS) | 1 | 4 | 0.491% |
| 50 (LP-SS+LP-WUS) | 2 | 4 | 0.920% |
| 50 (LP-SS+LP-WUS) | 4 | 4 | 1.777% |

## Annex 8.5.4	[8A-16]

| LP-WUS overhead |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Note | IDRX, 20Groups, 10kbps | IDRX, 20Groups, 5kbps | IDRX, 100 Groups, 10kbps | IDRX, 100 Groups, 5kbps | eDRX, 20Groups, 10kbps | eDRX, 20Groups, 5kbps | eDRX, 100Groups, 10kbps | eDRX, 100Groups, 5kbps |
| System config | system BW (MHz) | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 |
|  | total number of Idle UEs in the cell | 20 | 20 | 100 | 100 | 20 | 20 | 100 | 100 |
| LP-WUS config | # of info bits | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
|  | data rate (kbps) | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |
|  | LP-WUS time duration (ms) | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 |
|  | bandwidth (MHz) | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |
|  | Monitoring occasion periodicity (ms) | 1280 | 1280 | 1280 | 1280 | 82900 | 82900 | 82900 | 82900 |
|  | LP-WUS resource per group (ms*MHz) | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
|  | WUS group size | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |
|  | number of LP-WUS groups | 20 | 20 | 100 | 100 | 20 | 20 | 100 | 100 |
| Page Traffic | per group page inter arrival time (sec) | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
|  | R_G per group per peridocity | 0.7220 | 0.7220 | 0.7220 | 0.7220 | 1.0000 | 1.0000 | 1.0000 | 1.0000 |
|  | # of LP-WUS transmission per sec | 11.2807 | 11.2807 | 56.4033 | 56.4033 | 0.2413 | 0.2413 | 1.2063 | 1.2063 |
| System Resources | total system resources per LP-WUS periodicity (ms*MHz) | 25600 | 25600 | 25600 | 25600 | 1658000 | 1658000 | 1658000 | 1658000 |
| LP-WUS resources | actually used monitoring occasion resources used for LP-WUS tx per group per periodicity (ms*MHz) | 2.17 | 2.17 | 2.17 | 2.17 | 3.00 | 3.00 | 3.00 | 3.00 |
|  | total LP-WUS resources per LP-WUS periodicity | 43.32 | 43.32 | 216.59 | 216.59 | 60.00 | 60.00 | 300.00 | 300.00 |
|  | LP-WUS overhead (%) | 0.1692% | 0.1692% | 0.8461% | 0.8461% | 0.0036% | 0.0036% | 0.0181% | 0.0181% |

| LP-SS overhead |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Note | 10bits, periodicity = 1000ms | 10bits, periodicity = 500ms | 10bits, periodicity = 200ms | 15bits, periodicity = 1000ms | 15bits, periodicity = 500ms | 15bits, periodicity = 200ms |
| System config | system BW (MHz) | 20 | 20 | 20 | 20 | 20 | 20 |
| LP-SS config | # of info bits | 10 | 10 | 10 | 15 | 15 | 15 |
|  | data rate (kbps) | 10 | 10 | 10 | 10 | 10 | 10 |
|  | LP-WUS time duration (ms) | 1 | 1 | 1 | 1.5 | 1.5 | 1.5 |
|  | bandwidth (MHz) | 5 | 5 | 5 | 5 | 5 | 5 |
|  | Monitoring occasion periodicity (ms) | 1000 | 500 | 200 | 1000 | 500 | 200 |
|  | LP-SS resource per periodicity (ms*MHz) | 5 | 5 | 5 | 7.5 | 7.5 | 7.5 |
| Resources | total system resources per LP-WUS periodicity (ms*MHz) | 20000 | 10000 | 4000 | 20000 | 10000 | 4000 |
| LP-SS resources | # of LP-SS repetition per LP-SS burst | 4 | 4 | 4 | 4 | 4 | 4 |
|  | total LP-SS resoruces per periodicity | 20 | 20 | 20 | 30 | 30 | 30 |
|  | LP-SS overhead (%) | 0.10% | 0.20% | 0.50% | 0.15% | 0.30% | 0.75% |

## Annex 8.5.5	[8A-4]

For coverage and resource overhead evaluation, we consider a LP-WUS consisting of a payload field carrying16 information bits and a Frame Check Sequence (FCS) field carrying 8 CRC bits. The LP-SS is a 32-bit low density sequence (i.e., a sequence with lower number of 1 bits compared to 0 bits, 2 ON bits are considered in every 8 bits) occupying 4 OFDM symbols.

- LP-WUS-1: Waveform Option OOK-4 with M=2

- LP-WUS-2: Waveform Option FSK-2 with M=2

- LP-SS: Waveform Option OOK-4 with M=8

For both LP-WUS options, a SCS of 30 kHz, a 4.32 MHz LP-WUS bandwidth, and a 2.6 GHz Urban scenario (TDL-C 300) are considered. A 1/2 Manchester code is used for LP-WUS-1 resulting in an information data rate of 28kbps leading to payload+CRC occupying a total of 24 OFDM symbols. On the other hand, an information data rate of 56kbps is achieved by LP-WUS-2 leading to payload+CRC only occupying a total of 12 OFDM symbols. The coverage evaluation methodology in TR38.830 is used and the analysis is shown in Table 1 below.

Table 1: Link budget analysis for PUSCH (Msg.3) and LP-WUS/LP-SS

| Channel for evaluation | Msg.3 PUSCH | LP-WUS-1 | LP-WUS-2 | LP-SS |
| --- | --- | --- | --- | --- |
| Scenario and Carrier Frequency | FR1: Urban 2.6 GHz | FR1: Urban 2.6 GHz | FR1: Urban 2.6 GHz | FR1: Urban 2.6 GHz |
| Cell area reliability (%) | 90% | 99% | 99% | 99% |
| Transmitter |  |  |  |  |
| (1) Number of transmit antenna elements | 1 | 192 | 192 | 192 |
| (2) Number of transmit TxRUs | - | 64 | 64 | 64 |
| (2a) Number of transmit chains modelled in LLS | 1 | 2 | 2 | 2 |
| (3) Total transmit power (dBm) | 23 | 53 | 53 | 53 |
| Note: total transmit power for system bandwidth |  |  |  |  |
| (3a) System bandwidth for downlink, or occupied bandwidth for uplink (Hz) | 720000 | 100000000 | 100000000 | 100000000 |
| (3b) Power Spectrum Density = (3) - 10 log( (3a) / 1000000 ) (dBm/MHz) | 24.42667504 | 33 | 33 | 33 |
| (3c) Bandwidth used for the evaluated channel (Hz) | 720000 | 4320000 | 4320000 | 4320000 |
| (3bis) Total transmit power for occupied bandwidth = (3b) + 10 log ( (3c) / 1000000 ) (dBm) | 23 | 39.35483747 | 39.35483747 | 39.35483747 |
| (4) Total antenna gain at antenna gain component 3 & antenna gain component 4 of transmitter = (4a) – (4b) (dB) | 0 | 12.77121255 | 12.77121255 | 12.77121255 |
| (4a) Antenna gain at antenna gain component 3 & antenna gain component 4 of transmitter | 0 | 12.77121255 | 12.77121255 | 12.77121255 |
| = (4c) + 10 log ( (1) / (2) ) (dB) for downlink, and |  |  |  |  |
| = (4c) + 10 log ( (1) / (2a) ) (dB) for uplink |  |  |  |  |
| (4b) Antenna gain correction factor at antenna gain component 3 & antenna gain component 4 of transmitter (dB) | 0 | 0 | 0 | 0 |
| (4c) Gain of antenna element (dBi) | 0 | 8 | 8 | 8 |
| (5) Total antenna gain at antenna gain component 2 of transmitter = (5a) - (5b) (dB) | 0 | 15.05149978 | 15.05149978 | 15.05149978 |
| Note: zero for uplink |  |  |  |  |
| (5a) Antenna gain at antenna gain component 2 of transmitter = 10 log( (2)/(2a)) (dB) | 0 | 15.05149978 | 15.05149978 | 15.05149978 |
| Note: zero for uplink |  |  |  |  |
| (5b) Antenna gain correction factor at antenna gain component 2 of transmitter (dB) | 0 | 0 | 0 | 0 |
| Note: zero for uplink |  |  |  |  |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) | 1 | 3 | 3 | 3 |
| (9) EIRP = (3bis) + (4) + (5) – (8) dBm | 22 | 64.1775498 | 64.1775498 | 64.1775498 |
| Receiver |  |  |  |  |
| (10) Number of receive antenna elements | 192 | 1 | 1 | 1 |
| (10a) Number of receive TxRUs | 64 | - | - | - |
| Note: this row is void (empty) for downlink |  |  |  |  |
| (10b) Number of receive chains modelled in LLS | 2 | 1 | 1 | 1 |
| (11) Total antenna gain at antenna gain component 3 & antenna gain component 4 of receiver = (11a) - (11b) (dB) | 12.77121255 | 0 | 0 | 0 |
| (11a) Antenna gain at antenna gain component 3 & antenna gain component 4 of receiver | 12.77121255 | 0 | 0 | 0 |
| = (11c) + 10 log ( (10)/(10a) ) (dB) for uplink |  |  |  |  |
| = (11c) + 10 log ( (10)/(10b) ) (dB) for downlink |  |  |  |  |
| (11b) Antenna gain correction factor at antenna gain component 3 & antenna gain component 4 of receiver (dB) | 0 | 0 | 0 | 0 |
| (11c) Gain of antenna element (dBi) | 8 | 0 | 0 | 0 |
| (11bis) Total antenna gain at antenna gain component 2 of receiver = (11bis-a) - (11bis-b) (dB) | 15.05149978 | 0 | 0 | 0 |
| Note: zero for downlink |  |  |  |  |
| (11bis-a) Antenna gain at antenna gain component 2 of receiver = 10 log( (10a)/(10b)) (dB) | 15.05149978 | 0 | 0 | 0 |
| Note: zero for downlink |  |  |  |  |
| (11bis-b) Antenna gain correction factor at antenna gain component 2 of receiver (dB) | 0 | 0 | 0 | 0 |
| Note: zero for downlink |  |  |  |  |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) | 3 | 1 | 1 | 1 |
| (13) Receiver noise figure (dB) | 5 | 12 | 12 | 12 |
| (14) Thermal noise density (dBm/Hz) | -174 | -174 | -174 | -174 |
| (15) Receiver interference density (dBm/Hz) | -165.7 | -169.3 | -169.3 | -169.3 |
| (16) Total noise plus interference density = 10 log (10^(( (13) + (14))/10) + 10^((15)/10)) (dBm/Hz) | -164.0335231 | -161.2583889 | -161.2583889 | -161.2583889 |
| (18) Effective noise power = (16) + 10 log ((3c)) (dBm) | -105.4601981 | -94.90355143 | -94.90355143 | -94.90355143 |
| (19) Required SNR (dB) | -2.2 | 1.4 | 4.3 | 0.9 |
| (20) Receiver implementation margin (dB) | 2 | 2 | 2 | 2 |
| (21) H-ARQ gain (dB) | 0.5 | 0 | 0 | 0 |
| Note: Only applicable if HARQ is not considered in LLS |  |  |  |  |
| (22) Receiver sensitivity = (18) + (19) + (20) – (21) (dBm) | -106.1601981 | -91.50355143 | -88.60355143 | -92.00355143 |
| (22bis) MCL = (3bis) – (22) + (5) + (11bis) (dB) | 144.2116979 | 145.9098887 | 143.0098887 | 146.4098887 |
| (23) Hardware link budget, a.k.a. MIL = (9) + (11) + (11bis) − (12) − (22) (dB) | 152.9829104 | 154.6811012 | 151.7811012 | 155.1811012 |

The resource overhead incurred only by the LP-SS is evaluated in Table 2 where system bandwidths  and number of beams  are considered. Further, periodicities  are considered.

Table 2: Resource overhead for LP-SS

| System Bandwidth | 20 MHz |  |  | 100 MHz |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Number of Beams | 1 | 4 | 8 | 1 | 4 | 8 |
| LP-SS Periodicity |  |  |  |  |  |  |
| 320 ms | 0.0113% | 0.0450% | 0.0900% | 0.0023% | 0.0090% | 0.0180% |
| 640 ms | 0.0056% | 0.0225% | 0.0450% | 0.0011% | 0.0045% | 0.0090% |
| 1280 ms | 0.0028% | 0.0113% | 0.0225% | 0.0006% | 0.0023% | 0.0045% |
| 2560 ms | 0.0014% | 0.0056% | 0.0113% | 0.0003% | 0.0011% | 0.0023% |
| 5120 ms | 0.0007% | 0.0028% | 0.0056% | 0.0001% | 0.0006% | 0.0011% |
| 10240 ms | 0.0004% | 0.0014% | 0.0028% | 0.0001% | 0.0003% | 0.0006% |

Next, the resource overhead for LP-WUS-1 and LP-WUS-2 is evaluated in Table 3 for IoT use case according to the mMTC usage scenario with a connection density requirement of  devices/km2 as defined in the ITU-R M.2410 report on “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”. Additionally, paging arrival is considered according to the traffic model with 1 message/2 hours/device or 1 message/day/device as defined in the ITU-R M.2412 report on “Guidelines for evaluation of radio interface technologies for IMT-2020”. Further, a hexagonal network layout with inter-site distance (ISD) of 500m and 3 Transmission Reception Points (TRxPs) per site is considered. The average number of UEs to be simultaneously addressed in a LP-WUS is assumed to be . The results in Table 2 and Table 3 suggest that LP-SS/LP-WUS can be supported for IoT use case in urban deployments with coverage close to or exceeding that of PUSCH Msg3 at a resource overhead <1.9% considering 106 devices/km2 connection density, system bandwidth  MHz, number of transmission beams , and per UE paging arrival rate  1 message/2 hours.

Table 3: Resource overhead for LP-WUS considering 106 devices/km2 connection density

| System Bandwidth |  | 20 MHz |  |  |  | 100 MHz |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of Beams |  | 1 | 4 | 8 |  | 1 | 4 | 8 |
| Waveform | Traffic Model, Na |  |  |  |  |  |  |  |
| LP-WUS-1 | Model-1, Na=1 | 0.2165% | 0.8660% | 1.7321% |  | 0.0433% | 0.1732% | 0.3464% |
|  | Model-1, Na=4 | 0.0541% | 0.2165% | 0.4330% |  | 0.0108% | 0.0433% | 0.0866% |
|  | Model-2, Na=1 | 0.0180% | 0.0722% | 0.1443% |  | 0.0036% | 0.0144% | 0.0289% |
|  | Model-2, Na=4 | 0.0045% | 0.0180% | 0.0361% |  | 0.0009% | 0.0036% | 0.0072% |
| LP-WUS-2 | Model-1, Na=1 | 0.1083% | 0.4330% | 0.8660% |  | 0.0217% | 0.0866% | 0.1732% |
|  | Model-1, Na=4 | 0.0271% | 0.1083% | 0.2165% |  | 0.0054% | 0.0217% | 0.0433% |
|  | Model-2, Na=1 | 0.0090% | 0.0361% | 0.0722% |  | 0.0018% | 0.0072% | 0.0144% |
|  | Model-2, Na=4 | 0.0023% | 0.0090% | 0.0180% |  | 0.0005% | 0.0018% | 0.0036% |
|  | Model-1: 1 message/2 hours/device Model-2: 1 message/day/device |  |  |  |  |  |  |  |

## Annex 8.5.6	[8A-3]

We show network overhead impact with LP-WUS for different cases below. Evaluation details are given in RAN1#113 contribution [4].

- The total available resources are calculated assuming carrier bandwidth of 20 MHz (51 PRBs with 30 kHz SCS) and a TDD pattern with 4:1 ratio for DL:UL

- WUS payload:

  - 1bit,8bits,48bits

- WSU Coverage target

  - Msg3-PUSCH: {w/o retransmission, w/ two retransmissions}

  - PDCCH: {1Rx, AL16}, {2Rx, AL16}

  - Note: WUS structure to meet the coverage target is based on results in section 2.1

- Beam-sweeping assumptions:

  - No beam-sweeping

  - Beam-sweeping for both RRM and WUS monitoring assuming 8 beams

- Traffic

  - R_E = 0.1% (additional results for R_E = 1% included in the spreadsheet attached to this document)

  - Total 250 idle-mode UEs in a cell

- FAR assumption (FAR_target, N, T)

  - {0.1%, 1, 1.28s}

- Number of UEs per subgroup:

  - {1, 4}, 4 only applied for {1, 8} bit WUS payload

- Duty cycled WUS monitoring with 1.28s duty cycle is assumed for the results

- LP-SS overhead of 3slots per 1.28s assumed for case of P_WUR=0.5, no extra sync signal overhead assumed for P_WUR = {4, 10}.

Table 2.3.1 Network overhead for different cases with 1UE/subgroup

| 1UE per subgroup | Payload | 1bit |  |  |  | 8bit |  |  |  | 48bit |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R_E = 0.1% |  | Msg3 0rtx | Msg3 2rtx | PDCCH 1 Rx | PDCCH 2 Rx | Msg3 0rtx | Msg3 2rtx | PDCCH 1 Rx | PDCCH 2 Rx | Msg3 0rtx | Msg3 2rtx | PDCCH 1 Rx | PDCCH 2 Rx |
| No beam sweeping (single beam) | PWUR = 0.5 | 0,0427% | 0,0519% | 0,0549% | 0,0885% | 0,0580% | 0,1129% | 0,1434% | 0,2960% | 0,1251% | 0,3845% | 0,4547% | 1,2177% |
|  | PWUR = 4 | 0,00066% | 0,00083% | 0,00100% | 0,00183% | 0,00199% | 0,00199% | 0,00199% | 0,00249% |  |  |  |  |
|  | PWUR = 10 | 0,00066% | 0,00083% | 0,00100% | 0,00183% | 0,00199% | 0,00199% | 0,00199% | 0,00249% |  |  |  |  |
| Beam sweeping on LP-SS +LP-WUS (eight beams) | PWUR = 0.5 | 0,3418% | 0,4150% | 0,4395% | 0,7080% | 0,4639% | 0,9033% | 1,1475% | 2,3682% | 1,0010% | 3,0762% | ** | ** |
|  | PWUR = 4 | 0,00532% | 0,00664% | 0,00797% | 0,01462% | 0,01595% | 0,01595% | 0,01595% | 0,01993% |  |  |  |  |
|  | PWUR = 10 | 0,00532% | 0,00664% | 0,00797% | 0,01462% | 0,01595% | 0,01595% | 0,01595% | 0,01993% |  |  |  |  |
| ** Due to long WUS duration, beam-sweeping for LP-WUS with 48bits payload will be longer than paging cycle 1.28s Baseline overhead based on PDCCH, AL16, 2 OFDM symbols with 8 beams is 0.012%. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2.3.2 Network overhead for different cases with 4UE/subgroup

| 4UE per subgroup | Payload | 1bit |  |  |  | 8bit |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R_E = 0.1% |  | Msg3 0rtx | Msg3 2rtx | PDCCH 1 Rx | PDCCH 2 Rx | Msg3 0rtx | Msg3 2rtx | PDCCH 1 Rx | PDCCH 2 Rx |
| No beam sweeping (single beam) | PWUR = 0.5 | 0,0427% | 0,0519% | 0,0549% | 0,0885% | 0,0580% | 0,1129% | 0,1434% | 0,2960% |
|  | PWUR = 4 | 0,00066% | 0,00083% | 0,00100% | 0,00183% | 0,00199% | 0,00199% | 0,00199% | 0,00249% |
|  | PWUR = 10 | 0,00066% | 0,00083% | 0,00100% | 0,00183% | 0,00199% | 0,00199% | 0,00199% | 0,00249% |
| Beam sweeping on LP-SS +LP-WUS (eight beams) | PWUR = 0.5 | 0,3418% | 0,4150% | 0,4395% | 0,7080% | 0,4639% | 0,9033% | 1,1475% | 2,3682% |
|  | PWUR = 4 | 0,00532% | 0,00664% | 0,00797% | 0,01462% | 0,01595% | 0,01595% | 0,01595% | 0,01993% |
|  | PWUR = 10 | 0,00532% | 0,00664% | 0,00797% | 0,01462% | 0,01595% | 0,01595% | 0,01595% | 0,01993% |
| Baseline overhead based on PDCCH, AL16, 2 OFDM symbols with 8 beams is 0.012%. |  |  |  |  |  |  |  |  |  |

Overall following observations can be made on NW overhead based on the results in this and previous contributions.

For WUS to match PDCCH 2Rx coverage, following overhead values for different WUS candidates (including sync resource overhead) are observed from the evaluations (250 Idle UEs/cell assumed):

  - For per UE paging rate of 0.1% (1%)

    - Baseline overhead for paging PDCCH is 0.012% (0.12%)

    - For OFDM (SSS-sequence-based) WUS with 8-bit payload (4UEs/subgroup): ~1.65x (~1.45x) higher overhead compared to baseline is required

    - For OOK-WUS with 8 bit payload (4UEs/subgroup): ~196x (~154x) higher overhead compared to baseline is required

    - For OOK-WUS with 48 bit payload (no subgrouping): ~830x higher overhead compared to baseline (for 1% UE paging rate, overhead is prohibitively high)

Following general observations can be made for network overhead from the evaluations.

  - Regarding WUR on power (P_WUR)

    - Overhead for OOK-based-WUS (e.g., P_WUR=0.5) is roughly ~50-100x higher compared to that of OFDM-based-WUS (e.g., P_WUR=4, 10) as the OOK-based WUS duration required to achieve the same coverage target as OFDM-based WUS is much higher and there is a need for additional periodic LP-SS transmissions.

  - Regarding WUS payload

    - Overhead for OOK-based WUS with 48-bit payload is significantly higher than that of OOK-based WUS with 1- or 8-bit payload (~ 5x higher for single beam and eight beams).

    - Overhead for 8-bit WUS is slightly higher than that of 1-bit WUS.

  - Regarding number of UEs per subgroup

    - Overhead with 4 UEs per subgroup is similar to that of 1 UE per subgroup as the assumed low paging rate results in low probability of having UEs being paged in the same group in the same paging cycle.

## Annex 8.5.7	[8A-9]

Table 10: System overhead for LP-WUS for RRC IDLE/INACTIVE

| Signal | # of info bits carried by signal | Overhead (%) | T resources per beam | F resources | SCS | number of paged UE per second or signal periodicity | number of UE per group | Number of LP-WUS transmissions per second | System configuration, e.g., BW and etc. | the target channel | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LP-WUS | 48 | 0.0720% | 48 symbols | 5.04MHz | 30kHz | 10 | 1 | 10 | 20MHz, 1beam | Msg.3 | OOK-4 when 8 OFDM symbols are used. OOK-1 when 64 OFDM symbols are used |
| LP-WUS | 48 | 0.2808% | 48 symbols | 5.04MHz | 30kHz | 39 | 1 | 39 | 20MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.3600% | 48 symbols | 5.04MHz | 30kHz | 50 | 1 | 50 | 20MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.5760% | 48 symbols | 5.04MHz | 30kHz | 10 | 1 | 10 | 20MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 2.2464% | 48 symbols | 5.04MHz | 30kHz | 39 | 1 | 39 | 20MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 2.8800% | 48 symbols | 5.04MHz | 30kHz | 50 | 1 | 50 | 20MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.0144% | 48 symbols | 5.04MHz | 30kHz | 10 | 1 | 10 | 100MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.0562% | 48 symbols | 5.04MHz | 30kHz | 39 | 1 | 39 | 100MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.0720% | 48 symbols | 5.04MHz | 30kHz | 50 | 1 | 50 | 100MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.1152% | 48 symbols | 5.04MHz | 30kHz | 10 | 1 | 10 | 100MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.4493% | 48 symbols | 5.04MHz | 30kHz | 39 | 1 | 39 | 100MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.5760% | 48 symbols | 5.04MHz | 30kHz | 50 | 1 | 50 | 100MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.0072% | 48 symbols | 5.04MHz | 30kHz | 10 | 1 | 1 | 20MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.0281% | 48 symbols | 5.04MHz | 30kHz | 39 | 1 | 3.9 | 20MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.0360% | 48 symbols | 5.04MHz | 30kHz | 50 | 1 | 5 | 20MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.0576% | 48 symbols | 5.04MHz | 30kHz | 10 | 1 | 1 | 20MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.2246% | 48 symbols | 5.04MHz | 30kHz | 39 | 1 | 3.9 | 20MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.2880% | 48 symbols | 5.04MHz | 30kHz | 50 | 1 | 5 | 20MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.0014% | 48 symbols | 5.04MHz | 30kHz | 10 | 1 | 1 | 100MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.0056% | 48 symbols | 5.04MHz | 30kHz | 39 | 1 | 3.9 | 100MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.0072% | 48 symbols | 5.04MHz | 30kHz | 50 | 1 | 5 | 100MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.0115% | 48 symbols | 5.04MHz | 30kHz | 10 | 1 | 1 | 100MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.0449% | 48 symbols | 5.04MHz | 30kHz | 39 | 1 | 3.9 | 100MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.0576% | 48 symbols | 5.04MHz | 30kHz | 50 | 1 | 5 | 100MHz, 1beam | Msg.3 |  |

Annex X:	Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-10 | RAN1#110bis | R1-2208666 |  |  |  | Baseline TR skeleton | 0.0.0 |
| 2022-10 | RAN1#110bis | R1-2210430 |  |  |  | Baseline TR skeleton after discussion | 0.0.1 |
| 2023-02 | RAN1#112 | R1-2302236 |  |  |  | Update TR and incorporating agreements for evaluation and receiver architecture from RAN1#110bis and RAN1#111 | 0.1.0 |
| 2023-05 | RAN1#113 | R1-2305954 |  |  |  | Update TR and incorporates agreements from RAN1#112 and RAN1#112bis | 0.2.0 |
| 2023-08 | RAN1#114 | R1-2308386 |  |  |  | - Implemented all the agreements until RAN1#113 - Resolved FFS issues according to the consensus in offline discussion. - Other format and editorial issues. | 0.3.0 |
| 2023-08 | RAN1#114 | R1-2308729 |  |  |  | Incorporates agreements after RAN1#114 | 0.4.0 |
| 2023-09 | RAN#101 | RP-231814 |  |  |  | Version for information to RAN | 1.0.0 |