| 3GPP TR 38.914 V0.1.1 (2025-09) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on 6G Scenarios and Requirements; (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	8

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	9

4	Deployment scenarios	10

4.0	General	10

4.1	Indoor hotspot	10

4.2	Dense urban	11

4.3	Rural	12

4.4	Urban macro	13

4.5	Sub-Urban macro	14

4.6	High speed	14

4.7	Extreme long distance coverage in low density area	15

4.8	Urban coverage for massive connection	15

4.9	Air-to-Ground Scenario	15

4.10	Non-Terrestrial Network	15

4.11	[Urban grid]	16

4.12	[Highway Scenario]	16

5	Requirements and key technical principles	16

5.1	Key performance indicators	16

5.1.0	General	16

5.1.1	Peak data rate	16

5.1.2	Peak spectral efficiency	16

5.1.3	User experienced data rate	17

5.1.4	5th percentile user spectral efficiency	17

5.1.5	Average spectral efficiency	18

5.1.6	Area traffic capacity	18

5.1.7	User plane latency	19

5.1.8	Control plane latency	19

5.1.9	Connection density	19

5.1.10	Reliability	19

5.1.11	Mobility	20

5.1.12	Mobility interruption time	20

5.1.13	Bandwidth	20

5.1.14	Positioning	21

5.1.15	Energy efficiency	21

5.1.16	Composite requirement	21

5.1.17	Sensing	21

5.1.18	AI	21

5.2	Requirements for architecture and migration	21

5.3	Operational requirements	21

5.3.1	Spectrum	21

5.3.2	Diverse device types	21

5.3.3	RAN sharing	21

5.3.4	Ease of operation and Self Organization	21

5.3.5	Network Resilience	21

5.3.6	[Service awareness in RAN]	21

5.4	Requirements of New and Existing Services	21

5.5	Key technical principles	21

5.6	Testing and Conformance requirements	21

Annex A (informative): Change history	22

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

In June 2023, ITU-R Working Party 5D (WP5D) completed the development of 6G Framework in ITU-R M.2160 [2]. Compared with IMT-2020, the 6G framework presents:

-	Expanded and new usage scenarios, and

-	Enhanced and new capabilities.

In December 2024, ITU WP 5D invited External Organizations to provide their inputs on the minimum technical performance requirements (TPR) for IMT-2030 radio interface technologies, including but not limited to,

-	Proposed candidate items for minimum technical performance requirements based on M.2160 Usage Scenarios and Capabilities, including necessary background information and justification, which are requested to be provided preferably by WP 5D #48 (4-13 February 2025).

-	Proposed associated target values for the above candidate items for the minimum technical performance requirements which are requested to be provided preferably by WP 5D #49 (24 June – 3 July 2025, TBC)

Therefore, one purpose of this study is to investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160 [2], and the associated target values for the identified minimum TPRs. The output of this study is expected to be shared with ITU-R in the form of official liaisons.

5G has revolutionised the way we connect, offering unprecedented throughput, low latency, and the capacity to handle a vast number of connected devices, thus driving innovation in the consumer market and various verticals such as healthcare, automotive, industrial automation, satellite and smart cities. Society is now very much dependent on mobile communications. Business momentum triggered by 5G is contributing to the digital transformation of the society, and frontiers of network capabilities are expanding. Networks are being rolled out that include many of the 5G planned capabilities. The lessons learnt from the development, deployment and operation of 5G networks around the world (e.g. system complexity and large number of migration options) will be considered to ensure that 6G will support future opportunities, e.g. based on new technology trends that enable continuous innovation.

6G aims to support societal advancements and to bring value to society in the 2030s and beyond in secure, resilient, environmentally and economically sustainable ways. In addition to providing diverse and novel 6G services, other considerations are needed, e.g. CAPEX/OPEX reduction, improvement of overall 3GPP system performance, and migration from and interworking with 5G aspects.

Therefore, this study aims to identify typical usage scenarios for 6G radio technology and the required capabilities for each scenario.The study will also translate these required capabilities into technical metrics that can be evaluated from a RAN perspective.

1	Scope

The present document reports on the scenarios and requirements for 6G Radio, which can be used as not only guidance to the technical work in 3GPP RAN WGs but also input for ITU-R when developing IMT-2030 technical performance requirements.

The objectives of the study item are:

-	investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs.

  - -	The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN

-	Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors.

-	Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services.

-	Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate.

-	Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least:

  - -	Ensure appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting

  - -	Energy efficiency and energy saving: both for network and device

  - -	Enhanced spectral efficiency

  - -	Enhanced overall coverage, focus on cell-edge performance and UL coverage

  - -	Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz

  - -	Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band

  - -	Target scalable and forward compatible design for diverse device types

  - -	Improved spectrum utilization and operations taking into account diverse spectrum allocations

  - -	Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs

  - -	Aim at a harmonized 6G Radio design for TN and NTN, including their integration

  - -	System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

-	Define a time plan and steer work as appropriate for the RAN WGs during the 6G WG SI to deliver high-level decisions at least on the following areas:

  - -	Fundamental 6G radio design aspects: waveform, numerology, channel coding, etc…

  - -	Overall high-level aspects of 5G to 6G migration

  - -	RAN architecture and interfaces, including RAN-Core interface

  - -	Coordination of 6G AI/ML framework

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Recommendation ITU-R M.2160-0 (11/2023), “Framework and overall objectives of the future development of IMT for 2030 and beyond”.

[3]	Report ITU-R M.[IMT-2030. TECH PERF REQ]

[4]	Report ITU-R M.[IMT-2030.EVAL]

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

Carea	area traffic capacity

ρ	TRxP density (TRxP/m2)

M	number of TRxPs

N	number of users

Q	number of bands

ri	normalized user throughput of user i

R	total peak data rate

Ri (T)		number of correctly received bits by user i (downlink) or from user i (uplink)

Ri (Ti)	number of correctly received bits of user i

Ruser	user experienced data rate

Rp	peak data rate

SEavg	average spectral efficiency

SEp	peak spectral efficiency in a band

SEpi	peak spectral efficiency in a component band i

SEuser	5th percentile user spectral efficiency

T		time over which the data bits are received

Ti	active session time for user i

W	channel bandwidth

Wi	bandwidth for a component band i

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

AI	Artificial Intelligence

BS	Base Station

CAPEX	Capital Expenditure

CDF	Cumulative Distribution Function

DL	Downlink

HRLLC	Hyper Reliable and Low Latency Communication

IC	Immersive Communication

IMT	International Mobile Telecommunications

IP	Internet Protocol

ISD	Inter-Site Distance

ITU	International Telecommunication Union

ITU-R	International Telecommunication Union Radiocommunication Sector

KPI	Key Performance Indicator

OPEX	Operational Expenditure

PDU	Protocol Data Unit

QoS	Quality of Service

RAN	Radio Access Network

RF	Radio Frequency

RIT	Radio Interface Technology

SDU	Service Data Unit

SRIT	Set of Radio Interface Technologies

TR	Technical Report

TRxP	Transmission and Reception Point

Tx	Transmitter

UE	User Equipment

UL	Uplink

WG	Working Group

4	Deployment scenarios

Editor note: Sensing related aspects will be included in section 4

Editor note: More deployment scenarios can be added

4.0	General

High-level descriptions on deployment scenarios including carrier frequency, aggregated system bandwidth, network layout / ISD / Orbit, BS / UE antenna elements, UE distribution / speed and service profile are proposed in this TR. It is assumed that more detailed attributes and simulation parameters, for example, the channel model, BS / UE Tx power, number of antenna ports, etc. should be defined in a different TR.

4.1	Indoor hotspot

The indoor hotspot deployment scenario focuses on small coverage per site/TRxP (transmission and reception point) and high user throughput or user density in buildings. The key characteristics of this deployment scenario are high capacity, high user density and consistent user experience indoor.

Some of its attributes are listed in Table 4.1.

Table 4.1: Attributes for indoor hotspot

| Attributes | Values or assumptions |  |
| --- | --- | --- |
| Carrier Frequency NOTE1 | Around 4 GHz Around 7 GHz Around 4GHz+Around 7GHz Around 30 GHz |  |
| Aggregated system bandwidth NOTE2 | Around 4 GHz: Up to [200] MHz (DL+UL) NOTE3 Around 7 GHz: Up to [400] MHz (DL+UL) NOTE3 Around 30 GHz: [1]GHz (DL+UL) NOTE3 |  |
| Layout | Single layer: - Indoor floor (Open office) |  |
| ISD | 20m for around 30GHz TBD on other carrier frequencies (Equivalent to [12]TRxPs per 120m x 50m) |  |
| BS antenna elements | TBD |  |
| UE antenna elements | TBD |  |
| User distribution and UE speed | 100% Indoor, 3km/h, [10] users per TRxP NOTE4 |  |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |  |
| NOTE 1: TBD NOTE 2: TBD NOTE 3: TBD NOTE 4: TBD |  |

4.2	Dense urban

The dense urban microcellular deployment scenario focuses on macro TRxPs with or without micro TRxPs and high user densities and traffic loads in city centres and dense urban areas. The key characteristics of this deployment scenario are high traffic loads, outdoor and outdoor-to-indoor coverage. This scenario will be interference-limited, using macro TRxPs with or without micro TRxPs. A continuous cellular layout and the associated interference shall be assumed.

Some of its attributes are listed in Table 4.2.

Table 4.2: Attributes for dense urban

| Attributes | Values or assumptions |  |
| --- | --- | --- |
| Carrier Frequency NOTE1 | Around 2GHz Around 4 GHz Around 7 GHz Around 4 GHz + Around 30GHz Around 4 GHz + Around 7 GHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz] |  |
| Aggregated system bandwidth NOTE2 | Around 700 MHz: Up to [60] MHz (DL+UL) Around 2GHz: Up to [120] MHz (DL+UL) Around 4 GHz: Up to [200] MHz (DL+UL) Around 7 GHz: Up to [400]MHz (DL+UL) Around 30 GHz: [1]GHz (DL+UL) |  |
| Layout | Single layer: - Hex. Grid Two layers for Around 4 GHz + Around 30 GHz: - Macro layer: Hex. Grid, 4GHz - Micro layer: Random drop, 30GHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz ([single layer or two layers])]:TBD |  |
| ISD | Macro: 200m Micro: 3 micro TRxPs per macro TRxP Micro: [100]m All micro TRxPs are all outdoor |  |
| BS antenna elements | TBD |  |
| UE antenna elements | TBD |  |
| User distribution and UE speed | Step1: Uniform/macro TRxP, 10 users per TRxP Step2: Uniform/macro TRxP + Clustered/micro TRxP, 10 users per TRxP [10] users per TRxP with single-layer only 80% indoor (3km/h), 20% outdoor (30km/h) |  |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |  |
| NOTE 1: TBD NOTE 2: TBD |  |

4.3	Rural

The rural deployment scenario focuses on larger and continuous coverage. The key characteristics of this scenario are continuous wide area coverage supporting high speed vehicles. This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

Some of its attributes are listed in Table 4.3.

Table 4.3: Attributes for rural scenario

| Attributes | Values or assumptions |  |
| --- | --- | --- |
| Carrier Frequency | Around 700 MHz (for ISD 1 or ISD 2) Around 4 GHz (for ISD 1) Around 700 MHz + Around 2 GHz (for ISD 2) Around 7GHz (ISD 1) Around 700MHz + Around 7GHz (for ISD 1) |  |
| Aggregated system bandwidth | Around 700 MHz: Up to [60] MHz (DL+UL) Around 2GHz: Up to [120] MHz (DL+UL) Around 4 GHz: Up to [200] MHz (DL+UL) Around 7GHz: Up to [400] MHz (DL+UL) |  |
| Layout | Single layer: - Hex. Grid |  |
| ISD | ISD 1: 1732m ISD 2: 5000m [ISD 3: 7500m assuming 700MHz] |  |
| BS antenna elements | TBD |  |
| UE antenna elements | TBD |  |
| User distribution and UE speed | [15% outdoor vehicles (120km/h), 15% outdoor (3 km/h) and 70% indoor (3 km/h), or 50% outdoor vehicles (120km/h), 50% indoor (3 km/h)] [10] users per TRxP |  |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |  |

4.4	Urban macro

The urban macro deployment scenario focuses on large cells and continuous coverage. The key characteristics of this scenario are continuous and ubiquitous coverage in urban areas. This scenario will be interference-limited, using macro TRxPs (i.e. radio access points above rooftop level).

Some of its attributes are listed in Table 4.4.

Table 4.4: Attributes for urban macro

| Attributes | Values or assumptions |  |
| --- | --- | --- |
| Carrier Frequency | Around 2GHz Around 4 GHz Around 7 GHz Around 4 GHz + Around 30GHz Around 4 GHz + Around 7 GHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz] |  |
| Aggregated system bandwidth | Around 700 MHz: Up to [60] MHz (DL+UL) Around 2GHz: Up to [120] MHz (DL+UL) Around 4 GHz: Up to [200] MHz (DL+UL) Around 7 GHz: Up to [400]MHz (DL+UL) Around 30 GHz: [1]GHz (DL+UL) |  |
| Layout | Single layer: - Hex. Grid Two layers for Around 4 GHz + Around 30 GHz: - Macro layer: Hex. Grid, 4GHz - Micro layer: Random drop, 30GHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz ([single layer or two layers])]:TBD |  |
| ISD | Macro: 500m [TBD on micro layout] |  |
| BS antenna elements | TBD |  |
| UE antenna elements | TBD |  |
| User distribution and UE speed | TBD [10] users per TRxP |  |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |  |

4.5	Sub-Urban macro

The sub urban deployment scenario focuses on large and continuous coverage. [The key characteristics of this scenario are continuous wide area coverage supporting pedestrian and vehicles, energy saving, positioning performance.] This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

Therefore this scenario deployment can be considered for 6G as well and the values or assumptions are described as Table 4.5.

Table 4.5: Attributes for Sub-Urban macro scenario

| Attributes | Values or assumptions |  |
| --- | --- | --- |
| Carrier Frequency | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 7GHz +Around 4GHz Around 2GHz+ Around 700 MHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz] |  |
| System bandwidth | Around 700 MHz: Up to [60] MHz (DL+UL) Around 2GHz: Up to [120] MHz (DL+UL) Around 4 GHz: Up to [200] MHz (DL+UL) Around 7 GHz: Up to [400]MHz (DL+UL) |  |
| Layout | Single layer Hex. Grid [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz ([single layer or two layers])]:TBD |  |
| ISD | ISD 1: 1299m ISD 2: 1732m |  |
| TRP antenna elements | TBD |  |
| UE antenna elements | TBD |  |
| User distribution and UE speed | [10% Outdoor pedestrian: 3km/h, 10% Outdoor in cars: 40km/h, 80% Indoor in houses: 3km/h] [10] users per TRxP |  |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |  |

4.6	High speed

Editor note: Highway scenario will be considered and whether highway scenario is merged to section 4.6 can be further discussed

Editor note: airborne will not be captured in section 4.6.

4.7	Extreme long distance coverage in low density area

The extreme Long Range deployment scenario is defined to allow for the provision of services for very large areas with low density of users whether they are humans and machines (e.g. wilderness, river or water, remote village or island areas, etc). The key characteristics of this scenario are Macro cells with very large area coverage supporting [at least] basic data speeds and voice services, with low to moderate user throughput and low user density.

Table 4.7: Attributes for extreme rural

| Attributes | Values or assumptions |  |
| --- | --- | --- |
| Carrier Frequency | Around 700 MHz |  |
| System Bandwidth | [20] MHz (DL+UL) |  |
| Layout |  |  |
| Cell range |  |  |
| User density and UE speed |  |  |
| Traffic model |  |  |

4.8	Urban coverage for massive connection

Editor note: focusing on MTC

4.9	Air-to-Ground Scenario

The commercial Air to Ground deployment scenario is defined to allow for the provision of services for commercial aircraft to enable both humans and machines aboard the aircraft to initiate and receive mobile services. It is not for the establishment of airborne based base stations.

The key characteristics of this scenario are upward pointed Macro cells with very large area coverage supporting basic data and voice services, with moderate user throughput that are optimized for high altitude users that are travelling at very high speeds. The commercial airlines aircrafts are likely equipped with an aggregation point (e.g. Relay).

Some of the characteristics of this deployment scenario are listed in Table 4.9.

Table 4.9: Attributes for commercial Air to Ground Scenario

| Attributes | Values or assumptions |  |
| --- | --- | --- |
| Carrier Frequency | Macro + relay: for BS to relay: [4] GHz |
| System Bandwidth | [100] MHz (DL+UL) |
| Layout |  |
| Cell range |  |
| User density and UE speed |  |
| Traffic model |  |

4.10	Non-Terrestrial Network

Editor note: orbit type and payload type will be captured in section 4.10

4.11	[Urban grid]

4.12	[Highway Scenario]

Editor note: More deployment scenarios can be added

5	Requirements and key technical principles

5.1	Key performance indicators

Editor note: More KPIs can be added

5.1.0	General

Editor note:	Aspects related to IMT-2030 TPRs and 3GPP RAN KPIs will be included in this section

Editor note:	The KPIs captured in RAN#108 focus on ITU IMT-2030 TPRs. The definitions are subject to revision based on further discussion and progress in ITU

Editor note:	Evaluation methodology to be added for each KPI.

This section describes the definitions and requirements of all KPIs.

The KPI targets defined in this section as well as in Report ITU-R M.[IMT-2030. TECH PERF REQ] [3] shall be supported by next generation access technologies. The evaluation methodology defined in Report ITU-R M.[IMT-2030.EVAL] [4] shall be used to evaluate the targets specified in Report ITU-R M.[IMT-2030. TECH PERF REQ] [3].

The KPIs captured in RAN#108 focus on ITU IMT-2030 TPRs. The definitions are subject to revision based on further discussion and progress in ITU

5.1.1	Peak data rate

[Peak data rate is the maximum achievable data rate under ideal conditions (in bit/s), which is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times).

Peak data rate is defined for a single mobile station. In a single band, it is related to the peak spectral efficiency in that band. Let W denote the channel bandwidth and SEp denote the peak spectral efficiency in that band. Then the peak data rate Rp is given by:

Rp = W × SEp

Peak spectral efficiency and available bandwidth may have different values in different frequency ranges. In case bandwidth is aggregated across multiple bands, the peak data rate will be summed over the bands. Therefore, if bandwidth is aggregated across Q bands then the total peak data rate is

R=i=1Q  Wi × SEpi

where Wi and SEpi (i = 1, …, Q) are the component bandwidths and spectral efficiencies respectively.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for peak data rate are as follows:

–	Downlink peak data rate is TBD Gbit/s.

–	Uplink peak data rate is TBD Gbit/s.]

5.1.2	Peak spectral efficiency

Peak spectral efficiency is the maximum data rate under ideal conditions normalised by channel bandwidth (in bit/s/Hz), where the maximum data rate is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times).

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for peak spectral efficiencies are as follows:

–	Downlink peak spectral efficiency is TBD bit/s/Hz.

–	Uplink peak spectral efficiency is TBD bit/s/Hz.

These values were defined assuming an antenna configuration to enable TBD spatial layers (streams) in the downlink and TBD spatial layers (streams) in the uplink.

5.1.3	User experienced data rate

Editor note: name of this KPI may need to be changed

User experienced data rate is the 5% point of the cumulative distribution function (CDF) of the user throughput. User throughput (during active time) is defined as the number of correctly received bits, i.e. the number of bits contained in the service data units (SDUs) delivered to Layer 3, over a certain period of time.

For the evaluation of uplink user experienced data rate, use system level simulation with no bandwidth scaling taking UE maximum output power into account.

For the evaluation of downlink user experienced data rate, in case of one frequency band and one layer of transmission reception points (TRxP), the user experienced data rate could be derived from the 5th percentile user spectral efficiency, let W denote the channel bandwidth and SEuser denote the 5th percentile user spectral efficiency. Then the user experienced data rate, Ruser is given by:

Ruser = W × SEuser

In case bandwidth is aggregated across multiple bands (one or more TRxP layers), the user experienced data rate will be summed over the bands.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for the user experienced data rate are as follows in the Dense Urban – IC [test environment]:

–	Downlink user experienced data rate is TBD Mbit/s.

–	Uplink user experienced data rate is TBD Mbit/s.

5.1.4	5th percentile user spectral efficiency

The 5th percentile user spectral efficiency is the 5% point of the CDF of the normalized user throughput. The normalized user throughput is defined as the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time, divided by the channel bandwidth and is measured in bit/s/Hz.

The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor, where the effective bandwidth is the operating bandwidth normalized appropriately considering the uplink/downlink ratio.

With Ri (Ti) denoting the number of correctly received bits of user i, Ti the active session time for user i and W the channel bandwidth, the (normalized) user throughput of user i, ri, is defined according to the following equation:

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for 5th percentile user spectral efficiency for various [test environments] are summarized in Table 5.1.4-1.

Table 5.1.4-1: 5th percentile user spectral efficiency

| [Test Environment] | Downlink (bit/s/Hz) | Uplink (bit/s/Hz) |
| --- | --- | --- |
| Indoor Hotspot – IC | 0.9 | TBD |
| Dense Urban – IC | 0.675 | TBD |
| Rural – IC | TBD | TBD |

5.1.5	Average spectral efficiency

Average spectral efficiency (corresponds to "spectrum efficiency" in Recommendation ITU R M.2160 [2]) is the aggregate throughput of all users (the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time) divided by the channel bandwidth of a specific band divided by the number of TRxPs and is measured in bit/s/Hz/TRxP.

The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor, where the effective bandwidth is the operating bandwidth normalized appropriately considering the uplink/downlink ratio.

Let Ri (T) denote the number of correctly received bits by user i (downlink) or from user i (uplink) in a system comprising a user population of N users and M TRxPs. Furthermore, let W denote the channel bandwidth and T the time over which the data bits are received. The average spectral efficiency, SEavg is then defined according to the following equation:

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for average spectral efficiency for various [test environments] are summarized in Table 5.1.5-1.

Table 5.1.5-1: Average spectral efficiency

| [Test Environment] | Downlink (bit/s/Hz/TRxP) | Uplink (bit/s/Hz/TRxP) |
| --- | --- | --- |
| Indoor Hotspot – IC | 27 | TBD |
| Dense Urban – IC | 23.4 | TBD |
| Rural – IC | TBD | TBD |

5.1.6	Area traffic capacity

Area traffic capacity is the total traffic throughput served per geographic area (in Mbit/s/m2). The throughput is the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time.

This can be derived for a particular use case (or deployment scenario) of one frequency band and one TRxP layer, based on the achievable average spectral efficiency, network deployment (e.g. TRxP (site) density) and bandwidth.

Let W denote the channel bandwidth and ρ the TRxP density (TRxP/m2). The area traffic capacity Carea is related to average spectral efficiency SEavg through the following equation:

Carea = ρ × W × SEavg

In case bandwidth is aggregated across multiple bands, the area traffic capacity will be summed over the bands.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirement for area traffic capacity in downlink is TBD Mbit/s/m² in the Indoor Hotspot – IC [test environment].

5.1.7	User plane latency

User plane latency is the contribution of the radio network to the time from when the source sends a packet to when the destination receives it (in ms). It is defined as the one-way time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface in either uplink or downlink in the network for a given service in unloaded conditions, assuming the mobile station is in the active state.

This requirement is defined for the purpose of evaluation in the Immersive Communication and Hyper Reliable and Low Latency Communication usage scenarios.

The minimum requirements for user plane latency are:

-	4ms for IC

-	1ms for HRLLC

The above assumes unloaded conditions (i.e. a single user) for small IP packets (e.g. 0 byte payload + IP header), for both downlink and uplink.

5.1.8	Control plane latency

Control plane latency refers to the time to move from a battery efficient state (e.g., IDLE) to start of continuous data transfer (e.g., ACTIVE).

This requirement is defined for the purpose of evaluation in the Immersive Communication and Hyper Reliable and Low Latency Communication usage scenarios.

The minimum requirement for control plane latency is 20 ms.

Note: Proponents are encouraged to consider lower control plane latency, e.g. 10 ms.

5.1.9	Connection density

Connection density is the number of connections fulfilling a specific quality of service (QoS) per unit area (per km2).

Connection density should be achieved for a given bandwidth and a given number of TRxPs. The target QoS is to support delivery of a message of a certain size within a certain time and with a certain success probability.

This requirement is defined for the purpose of evaluation in the Massive Communication usage scenario.

The minimum requirement for connection density is 106 connections per km2.

5.1.10	Reliability

Reliability relates to the capability of transmitting a given amount of traffic within a predetermined time duration with high success probability.

Reliability is the success probability of transmitting a layer 2/3 packet within a required maximum time, which is the time it takes to deliver a data packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface at a certain channel quality.

This requirement is defined for the purpose of evaluation in the Hyper Reliable and Low Latency Communication usage scenario.

The minimum requirement for the reliability is 1-10-5 success probability of transmitting a layer 2 PDU (protocol data unit) of 32 bytes within 1 ms in channel quality of coverage edge for the Urban Macro-HRLLC [test environment], assuming small application data (e.g. 20 bytes application data + protocol overhead).

Proponents are encouraged to consider larger packet sizes, e.g. layer 2 PDU size of up to 100 bytes.

5.1.11	Mobility

Mobility is the maximum mobile station speed at which a defined QoS can be achieved (in km/h).

The following classes of mobility are defined:

-	Stationary: 0 km/h

-	Pedestrian: 0 km/h to 10 km/h

-	Vehicular: 10 km/h to 120 km/h

-	High speed vehicular: 120 km/h to 500 km/h

-	Airplane: 500 km/h to 1000/1200 km/h

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

Table 5.1.11-1 defines the mobility classes that shall be supported in the respective [test environments].

Table 5.1.11-1: Mobility classes

|  | [Test environments] for IC |  |  |
| --- | --- | --- | --- |
|  | Indoor Hotspot – IC | Dense Urban – IC | Rural – IC |
| Mobility classes supported |  |  |  |

A mobility class is supported if the traffic channel link data rate on the uplink, normalized by bandwidth, is as shown in Table 5.1.11-2. This assumes the user is moving at the maximum speed in that mobility class in each of the [test environments].

Table 5.1.11-2: Traffic channel link data rates normalized by bandwidth

| [Test environment] |  | Normalized traffic channel link data rate (bit/s/Hz) | Mobility (km/h) |
| --- | --- | --- | --- |
| Indoor Hotspot – IC |  | 2.25 | 10 |
| Dense Urban – IC |  | 1.68 | 30 |
| Rural – IC |  | [0.8~1.2] | 120 |
|  |  | [0.45~0.675] | 500 |

These values were defined assuming an antenna configuration as described in Report ITU-R M.[IMT-2030.EVAL] [4].

5.1.12	Mobility interruption time

[Mobility interruption time is the shortest time duration supported by the system during which a user terminal cannot exchange user plane packets with any base station during transitions.

The mobility interruption time includes the time required to execute any radio access network procedure, radio resource control signalling protocol, or other message exchanges between the mobile station and the radio access network, as applicable to the candidate RIT/SRIT.

This requirement is defined for the purpose of evaluation in the Immersive Communication and Hyper Reliable and Low Latency Communication usage scenarios.

The minimum requirement for mobility interruption time is TBD.]

5.1.13	Bandwidth

Bandwidth is the maximum aggregated system bandwidth. The bandwidth may be supported by single or multiple radio frequency (RF) carriers. The bandwidth capability of the RIT/SRIT is defined for the purpose of IMT-2030 evaluation.

The requirement for bandwidth is at least 400 MHz.

5.1.14	Positioning

5.1.15	Energy efficiency

5.1.16	Composite requirement

Editor note: KPI related to data rate, latency, reliability, capacity. Name of this KPI may need to be changed

5.1.17	Sensing

5.1.18	AI

Editor note: More KPIs can be added

5.2	Requirements for architecture and migration

Editor note: 6G RAN architecture, 5G-6G migration

5.3	Operational requirements

Editor note: Spectrum, co-existence, topology and other operational requirements

5.3.1	Spectrum

Editor note: e.g. spectrum and co-existence related

5.3.2	Diverse device types

5.3.3	RAN sharing

5.3.4	Ease of operation and Self Organization

5.3.5	Network Resilience

5.3.6	[Service awareness in RAN]

5.4	Requirements of New and Existing Services

5.5	Key technical principles

5.6	Testing and Conformance requirements

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-06 | RAN#108 | RP-251393 |  |  |  | Skeleton for Study on 6G Scenarios and Requirements | 0.0.1 |
| 2025-06 | RAN#108 | RP-251819 |  |  |  | Skeleton for Study on 6G Scenarios and Requirements as agreed by RAN #108 | 0.0.2 |
| 2025-06 | RAN#108 | RP-251873 |  |  |  | Updated TR including pCRs approved at RAN #108 (RP-251872, RP-251880); v0.1.0 was agreed by RAN #108 | 0.1.0 |
| 2025-09 | RAN #109 | RP-252120 |  |  |  | Updated TR as provided by MCC to align with 3GPP drafting rules | 0.1.1 |

-