| 3GPP TR 28.869 V2.0.0 (2025-12) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on cloud aspects of management and orchestration (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	10

3.1	Terms	10

3.2	Symbols	11

3.3	Abbreviations	11

4	Concepts and background	11

4.1	Background of VNF generic OAM functions	11

4.1.1	Overview	11

4.1.2	Summary of TR 28.834 use cases related to generic OAM functions	11

4.2	Terminology considerations	12

4.2.1	Terminology and concepts used in the present document	12

4.2.2	Terminology alignment with ETSI NFV	12

4.3	Relationship with other 3GPP SA5 work	12

4.4	Cloud-native design principles and their relevance to 3GPP OAM	12

4.4.1	Background on cloud-native design principles	12

4.4.2	Twelve-factor app	13

4.4.3	CNCF's cloud-native principles	13

4.4.4	Relevance of cloud-native design principles to 3GPP OAM	14

4.5	Cloud deployment types	14

5	Use cases, potential requirements, and potential solutions	15

5.1	Use of VNF generic OAM functions	15

5.1.1	Use case #1: Cloud-native VNF configuration management	15

5.1.1.1	Description	15

5.1.1.2	Potential requirements	15

5.1.1.3	Potential solutions	15

5.1.1.3.1	VNF Configuration Manager function	15

5.1.1.3.2	Network Configuration Manager function	16

5.1.1.3.3	Configuration Server	16

5.1.1.3.4	Using the existing 3GPP provisioning management service and ETSI NFV MANO	17

5.1.1.3.5	Proposed updates in the 3GPP management system	17

5.1.1.3.6	Using the existing 3GPP provisioning management service	17

5.1.1.4	Evaluation of solutions	18

5.1.2	Use case #2：Cloud-native VNF policy management	18

5.1.2.1	Description	18

5.1.2.2	Potential requirements	18

5.1.2.3	Potential solutions	18

5.1.2.3.1	Policy Agent	18

5.1.2.3.2	Use of existing 3GPP provisioning management service and ETSI NFV MANO	19

5.1.2.4	Evaluation of solutions	19

5.1.3	Use case #3: Cloud-native VNF Traffic management	19

5.1.3.1	Description	19

5.1.3.2	Potential requirements	20

5.1.3.3	Potential solutions	20

5.1.3.3.1	Traffic Enforcer function	20

5.1.3.4	Evaluation of solutions	20

5.1.4	Use case #4: Cloud-native VNF Upgrade	20

5.1.4.1	Description	20

5.1.4.2	Potential requirements	21

5.1.4.3	Potential solutions	21

5.1.4.3.1	Upgrade VNF function	21

5.1.4.3.2	Use of existing 3GPP provisioning management service and ETSI NFV MANO	22

5.1.4.4	Evaluation of solutions	22

5.1.5	Relationship between 3GPP Management System (SBMA) and ETSI NFV VNF generic OAM functions and NFV-MANO	22

5.2	Use of industry solutions for management of NF Deployments	24

5.2.1	Use case #1: 3GPP management architecture evolution to support LCM of NF Deployment instance	24

5.2.1.1	Description	24

5.2.1.2	Potential requirements	24

5.2.1.3	Potential solutions	24

5.2.1.4	Evaluation of solutions	25

5.2.2	Use case #2: data streaming for cloud-native NF deployment	25

5.2.2.1	Description	25

5.2.2.2	Potential requirements	25

5.2.2.3	Potential solutions	26

5.2.2.3.1	Solution #1: Management data streaming based on message bus	26

5.2.2.3.2	Solution #2: Management data streaming for cloud-native NF deployments	26

5.2.2.4	Evaluation of solutions	26

5.2.3	Use case #3: Creation of a NF Deployment instance	27

5.2.3.1	Description	27

5.2.3.2	Potential requirements	27

5.2.3.3	Potential solutions	27

5.2.3.3.1	Use of deployment management reference point	27

5.2.3.3.2	Use of deployment management reference point based on declarative descriptor	28

5.2.3.4	Evaluation of solutions	28

5.2.4	Use case #4: Modification of a NF Deployment instance	29

5.2.4.1	Description	29

5.2.4.2	Potential requirements	29

5.2.4.3	Potential solutions	29

5.2.4.3.1	Use of deployment management reference point	29

5.2.4.3.2	Use of deployment management reference point based on declarative descriptor	29

5.2.4.4	Evaluation of solutions	30

5.2.5	Use case #5: Termination of a NF deployment instance	30

5.2.5.1	Description	30

5.2.5.2	Potential requirements	31

5.2.5.3	Potential solutions	31

5.2.5.3.1	Use of deployment management reference point	31

5.2.5.3.2	Use of deployment management reference point based on declarative descriptor	31

5.2.5.4	Evaluation of solutions	32

5.2.6	Use case #6: Scaling of NF Deployment instance(s)	32

5.2.6.1	Description	32

5.2.6.2	Potential requirements	32

5.2.6.3	Potential solutions	33

5.2.6.3.1	Use of deployment management reference point	33

5.2.6.3.2	Use of deployment management reference point based on declarative descriptor	33

5.2.6.4	Evaluation of solutions	34

5.2.7	Use case #7: Observability for Network Functions in supporting cloud native	34

5.2.7.1	Description	34

5.2.7.2	Potential requirements	35

5.2.7.3	Potential solutions	35

5.2.7.4	Evaluation of solutions	36

5.3	Support of different cloud deployment scenarios	36

5.3.1	Use case #1: Placement of cloud native NFs	36

5.3.1.1	Description	36

5.3.1.2	Potential requirements	36

5.3.1.3	Potential solutions	36

5.3.1.4	Evaluation	36

6	Conclusions and recommendations	36

6.1	General conclusion	36

6.2	Data streaming for cloud-native NF deployments	37

6.3	Terminology and concepts	37

6.4	LCM of NF Deployment	37

6.5	Use of VNF generic OAM functions	37

6.6	Observability for Network Functions in supporting cloud native	38

Annex A: Example implementation of LCM for NF Deployments using Kubernetes Custom Resource Definitions	39

Annex B: Example implementations using management data streaming solution based on message bus	41

Annex C: Parallel streaming, scaling and resiliency aspects of using management data streaming solution based on message bus	42

Annex D: Example scenario using interactions with orchestration and management entity for the creation of NF Deployment instances	44

Annex E: Example scenario using interactions with orchestration and management entity for modification of NF Deployment instances	45

Annex F: Example scenario using interactions with orchestration and management entity for termination of NF Deployment instances	46

Annex G: Support for containerized VNF deployments in ETSI NFV	47

Annex H: Support for CNF deployments in ETSI NFV	49

H.1	Create CNF example using NFV-MANO	49

Annex I (informative): Change history	50

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document studies cloud aspects of management and orchestration of the 5GS; specifically, use of ETSI VNF generic OAM functions [2], use of industry solutions including ETSI NFV MANO, open source and other solutions for management (including life cycle management and other management aspects) of cloud native network functions, and support of different cloud deployment scenarios.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Technical Specification Group Services and System Aspects; Vocabulary for 3GPP Specifications".

[2]	ETSI GS NFV-IFA 049: "Network Functions Virtualisation (NFV) Release 5; Architectural Framework; VNF generic OAM functions and other Paas Services specification".

[3]	ETSI GR NFV-EVE 019: "Network Functions Virtualisation (NFV);Architectural Framework; Report on VNF generic OAM functions".

[4]	3GPP TR 28.834: "Technical Specification Group Services and System Aspects; Study on management of cloud-native Virtualized Network Functions (VNF)".

[5]	SP-230764: "New WID on Management of cloud-native Virtualized Network Functions".

[6]	3GPP TS 28.526: "Technical Specification Group Services and System Aspects; Telecommunication management; Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Procedures".

[7]	3GPP TS 28.531: "Technical Specification Group Services and System Aspects; Management and orchestration; Provisioning".

[8]	ETSI GS NFV-IFA 013 (V4.5.1) (2023-09): "Network Function Virtualisation (NFV); Release 4; Management and Orchestration; Os-Ma-nfvo reference point - Interface and Information Model Specification".

[9]	ETSI GS NFV-IFA 008 (V4.3.1) (2022-05): "Network Function Virtualisation (NFV); Release 4; Management and Orchestration; Ve-Vnfm reference point - Interface and Information Model Specification".

[10]	3GPP TS 28.532: "Technical Specification Group Services and System Aspects; Management and orchestration; Generic management services".

[11]	ETSI GR NFV 003 (V1.8.6): "Network Functions Virtualisation (NFV); Terminology for Main Concepts in NFV".

[12]	3GPP TS 28.555: "Technical Specification Group Services and System Aspects; Management and orchestration; Network policy management for 5G mobile networks; Stage 1".

[13]	ETSI GR NFV-IFA 023 (V3.1.1): "Network Functions Virtualisation (NFV); Management and Orchestration; Report on Policy Management in MANO; Release 3".

[14]	3GPP TS 28.552: "Technical Specification Group Services and System Aspects; Management and orchestration; 5G performance measurements".

[15]	3GPP TS 28.554: "Technical Specification Group Services and System Aspects; Management and orchestration; 5G end to end Key Performance Indicators (KPIs)".

[16]	3GPP TS 28.533: "Technical Specification Group Services and System Aspects; Management and orchestration; Architecture framework".

[17]	3GPP TS 28.541: "Technical Specification Group Services and System Aspects; Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[18]	ETSI GS NFV 006: "Network Functions Virtualisation (NFV) Release 4; Management and Orchestration; Architectural Framework Specification".

[19]	ETSI GS NFV-IFA 007: "Network Functions Virtualisation (NFV) Release 5; Management and Orchestration; Or-Vnfm reference point - Interface and Information Model Specification".

[20]	ETSI GS NFV-IFA 008: "Network Functions Virtualisation (NFV) Release 5; Management and Orchestration; Ve-Vnfm reference point - Interface and Information Model Specification".

[21]	ETSI GS NFV-IFA 010: "Network Functions Virtualisation (NFV) Release 5; Management and Orchestration; Functional requirements specification".

[22]	ETSI GS NFV-IFA 011: "Network Functions Virtualisation (NFV) Release 5; Management and Orchestration; VNF Descriptor and Packaging Specification".

[23]	ETSI GS NFV-IFA 013: "Network Functions Virtualisation (NFV) Release 5; Management and Orchestration; Os-Ma-nfvo reference point - Interface and Information Model Specification".

[24]	ETSI GS NFV-IFA 014: "Network Functions Virtualisation (NFV) Release 5; Management and Orchestration; Network Service Templates Specification".

[25]	ETSI GR NFV-IFA 029: "Network Functions Virtualisation (NFV) Release 3; Architecture; Report on the Enhancements of the NFV architecture towards "Cloud-native" and "PaaS".

[26]	ETSI GS NFV-IFA 036: "Network Functions Virtualisation (NFV) Release 5; Management and Orchestration; Requirements for service interfaces and object model for container cluster management and orchestration specification".

[27]	ETSI GR NFV-IFA 038: "Network Functions Virtualisation (NFV) Release 4; Architectural Framework; Report on network connectivity for container-based VNF".

[28]	ETSI GS NFV-IFA 040: "Network Functions Virtualisation (NFV) Release 5; Management and Orchestration; Requirements for service interfaces and object model for OS container management and orchestration specification".

[29]	ETSI GR NFV-IFA 043: "Network Functions Virtualisation (NFV) Release 5; Architectural Framework; Report on enhanced container networking".

[30]	ETSI GS NFV-SOL 001: "Network Functions Virtualisation (NFV) Release 5; Protocols and Data Models; NFV descriptors based on TOSCA specification".

[31]	ETSI GS NFV-SOL 002: "Network Functions Virtualisation (NFV) Release 5; Protocols and Data Models; RESTful protocols specification for the Ve-Vnfm Reference Point".

[32]	ETSI GS NFV-SOL 003: "Network Functions Virtualisation (NFV) Release 5; Protocols and Data Models; RESTful protocols specification for the Or-Vnfm Reference Point".

[33]	ETSI GS NFV-SOL 004: "Network Functions Virtualisation (NFV) Release 5; Protocols and Data Models; VNF Package and PNFD Archive specification".

[34]	ETSI GS NFV-SOL 005: "Network Functions Virtualisation (NFV) Release 5; Protocols and Data Models; RESTful protocols specification for the Os-Ma-nfvo Reference Point".

[35]	ETSI GS NFV-SOL 016: "Network Functions Virtualisation (NFV) Release 4; Protocols and Data Models; NFV-MANO procedures specification".

[36]	ETSI GS NFV-SOL 018: "Network Functions Virtualisation (NFV) Release 5; Protocols and Data Models; Profiling specification of protocol and data model solutions for OS Container management and orchestration".

[37]	ETSI GS NFV-SOL 020: "Network Functions Virtualisation (NFV) Release 5; Protocols and Data Models Specification of protocols and data models for Container Infrastructure Service Cluster Management".

[38]	ETSI GS NFV-IFA 027: "Network Functions Virtualisation (NFV) Release 5; Management and Orchestration; Performance Measurements Specification".

[39]	ETSI GR NFV-EVE 021: "Network Functions Virtualisation (NFV) Release 5; Evolution and Ecosystem; Report on energy efficiency aspects for NFV".

[40]	ETSI GS NFV-IFA 053: "Network Functions Virtualisation (NFV) Release 5; Management and Orchestration; Requirements and interface specification for Physical Infrastructure Management".

[41]	3GPP TS 23.501: "Technical Specification Group Services and System Aspects; System architecture for the 5G System (5GS); Stage 2".

[42]	CNCF Cloud Native Definition v1.1.

NOTE 1:	Available at https://github.com/cncf/toc/blob/main/DEFINITION.md.

[43]	"The twelve-factor app".

NOTE 2:	Available at https://12factor.net.

[44]	ETSI GR NFV-EVE 022: "Network Functions Virtualisation (NFV) Release 5; Architectural Framework; Report on VNF configuration".

[45]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[46]	"OpenTelemetry"

NOTE 3:	Available at https://opentelemetry.io/.

[47]	"Extend the Kubernetes API with CustomResourceDefinitions |Kubernetes".

NOTE 4:	Available at https://kubernetes.io/docs/tasks/extend-kubernetes/custom-resources/custom-resource-definitions/.

[48]	"Operator Pattern | Kubernetes".

NOTE 5:	Available at https://kubernetes.io/docs/concepts/extend-kubernetes/operator/.

[49]	3GPP TS 28.556: "Technical Specification Group Services and System Aspects; Management and orchestration; Network policy management for 5G mobile networks; Stage 2 and stage 3".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

VNF generic OAM function: defined in ETSI GS NFV-IFA 049 [2].

Virtualized Network Function: defined in ETSI GR NFV 003 [11].

Message bus: a mechanism for sending and receiving messages between distributed systems via a middleware messaging infrastructure.

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

CSP	Communication Service Provider

LCM	Life Cycle Management

# 4	Concepts and background

## 4.1	Background of VNF generic OAM functions

### 4.1.1	Overview

ETSI GR NFV-EVE 019 [3] defines and analyses the type of OAM functions for VNFs that can be generalized and be provided as a "generic function". It also describes potential solutions regarding the VNF generic OAM functions architectural framework and the interactions with the other entities in the NFV ecosystem.

3GPP TR 28.834 [4] "Study on management of cloud-native Virtualized Network Functions (VNF)" [4] in Release 18 studies potential use cases, requirements and solutions for the management of cloud-native virtualized network functions and the impacts on the 3GPP management system. It also includes some use cases related to VNF generic OAM functions, but the relevant specific solutions were not studied thoroughly due to the fact that the corresponding ETSI GS NFV-IFA 049 [2] was not published prior to the completion of the 3GPP TR 28.834 [4]. As a result, the scope of Rel-18 "WID on management of cloud-native Virtualized Network Functions" [5] did not include enhancements related to VNF generic OAM functions.

ETSI GS NFV-IFA 049 [2] specifies the VNF generic OAM functions framework and also specifies the functional requirements and the interface requirements for VNF generic OAM functions. Considering that ETSI GS NFV-IFA 049 [2] has been published, clause 5.1 of this Rel-19 TR studies the use of VNF generic OAM functions from the perspective of 3GPP management system.

As specified in ETSI GS NFV-IFA 049 [2], close mapping between VNF generic OAM functions and Platform as a Service (PaaS) Services exists. According to ETSI GS NFV-IFA 049 [2], any VNF generic OAM function can be understood as a PaaS Service of a specific type, i.e. a PaaS Service that provides and handles generic OAM functionality for VNFs. See ETSI GR NFV 003 [11] for a definition of the term PaaS Service.

### 4.1.2	Summary of TR 28.834 use cases related to generic OAM functions

In 3GPP TR 28.834 [4], clauses 5.1, 5.2 and 5.3 are the use cases related to generic OAM functions, which are about using the "VNF configuration manager function", "Traffic enforcer function", "VNF metrics aggregator function" and "VNF metrics analyser function".

In clause 5, the above use cases describe the use case scenarios of the interaction process between the generic OAM functions and the 3GPP management system based on the content of ETSI GR NFV-EVE 019 [3] and give some potential related requirements, while clause 6 provides a simple analysis and does not have specific solutions.

This study carries out further research on the use of generic OAM functions based on ETSI GS NFV-IFA 049 [2].

## 4.2	Terminology considerations

### 4.2.1	Terminology and concepts used in the present document

3GPP network function (NF) is specified in TS 23.501 [41], as follows:

Network Function: A 3GPP adopted or 3GPP defined processing function in a network, which has defined functional behaviour and 3GPP defined interfaces.

To support cloud native concepts in 3GPP, an NF Deployment represents the software deployment used to realize a part of, or the full function of, one or multiple 3GPP network functions. The use of NF Deployment is not limited by any specific virtualization technology, e.g. VM based, or container based.

A NF Deployment instance is a deployed software instance designed to run on cloud, deployed and managed using technologies and principles evolving in the cloud eco system. One fundamental cloud native principle is to partition systems into smaller, separately manageable parts. This allows for faster and more automated upgrades, improve operational efficiency and shorter time to market for new services.

The present study proposes, but does not limit, to use NF Deployment as concept and terminology in the present document. The instance(s) of NF Deployment is/are created, modified, or terminated through LCM related operations using an orchestration and management system.

NOTE 1:	The term "cloud native VNF" and "NF deployment" used in the present document are closely related, nevertheless there is no consensus on their definition and concepts used in this present document and are expected to be investigated in the normative phase.

NOTE 2:	The term "cloud native VNF" is used in clause 5.1 and "NF deployment" is used in clause 5.2 of the present document, these terms may all be updated in the normative phase.

### 4.2.2	Terminology alignment with ETSI NFV

In ETSI NFV according to ETSI GR NFV 003 [11]:

-	terms virtualization and cloudification are used interchangeably.

-	the terms "cloud-native VNF" and "cloud-native NF" are used interchangeably.

-	a containerized NF, a container-based NF, a containerized VNF, and a container-based VNF refer to the same concept (i.e. VNF whose software components are deployed within OS containers) and are used interchangeably.

-	the descriptor is VNFD, as per ETSI NFV specifications (see ETSI GS NFV-IFA 011 [22]).

The above relationships and terminology are considered for example by ETSI GS NFV-IFA 049 [2] when describing the use of VNF generic OAM functions.

EXAMPLE:	When describing that the Log aggregator supports the capability to collect different types of logs from different VNFs, both containerized VNFs and VM-based VNFs are considered.

## 4.3	Relationship with other 3GPP SA5 work

3GPP SA5 has begun studying aspects of cloud-native related to 3GPP's mobile network management in Release 18. In 3GPP TR 28.834 [4], there are some use cases and potential solutions based on the latest Release 4 specifications from ETSI NFV to support the cloud native VNF management by interacting with ETSI NFV's NFV-MANO, which resulted in normative work updating TS 28.526 [6], TS 28.531 [7] and TS 28.533 [16].

## 4.4	Cloud-native design principles and their relevance to 3GPP OAM

### 4.4.1	Background on cloud-native design principles

Cloud-native design principles are described and documented in various industry standards, frameworks, open-source communities and key organizations in the field of cloud computing but no single standardized definition exists for cloud-native design principles. Two relevant sources of cloud-native principles are "The twelve-factor app" [43] and the CNCF's "Cloud-native design principles" [42].

### 4.4.2	Twelve-factor app

Most descriptions of cloud-native design principles trace back to a set of design principles as described in "The twelve-factor app" [43]. These factors are:

1:	Codebase: One codebase tracked in revision control, many deploys

2:	Dependencies: Explicitly declare and isolate dependencies

3:	Configuration: Store config in the environment

4:	Backing services: Treat backing services as attached resources

5:	Build, release, and run: Strictly separate build and run stages

6:	Processes: Execute the app as one or more stateless processes

7:	Port binding: Export services via port binding

8:	Concurrency: Scale out via the process model

9:	Disposability: Maximize robustness with fast startup and graceful shutdown

10:	Dev/Prod parity: Keep development, staging, and production as similar as possible

11:	Logs: Treat logs as event streams

12:	Administrative processes: Run admin/management tasks as one-off processes

### 4.4.3	CNCF's cloud-native principles

Other industry-recognized cloud-native design principles specified in "Cloud-native design principles" [42] are:

-	Micro-services design.

-	Loosely coupled: implies that each micro-service composing a cloud-native application is a small independent deployable unit. This principle enables each micro-service to evolve independently of the other micro-services.

-	Containerization: Micro-services and their dependencies are packaged into containers that can run independently across all environments and host operating systems.

-	Repeatable deployment process: Since cloud-native applications are based on containers, the deployment process of cloud-native applications is reproducible, consistent, and thus repeatable (i.e. can be automated).

-	Immutable infrastructure: implies that the infrastructure hosting the cloud-native applications cannot be modified in place. Modifications are only possible by building new infrastructure templates and then rebuilding relevant infrastructure using those templates.

-	Declarative API: cloud-native applications use declarative APIs to define what to do (i.e. the desired state) instead of how to do it.

-	Observability: monitoring cloud-native applications through access to metrics, logs, and traces should be possible.

-	Resiliency: cloud-native applications should be able to tolerate failures by leveraging self-healing mechanisms (e.g. system restarts).

-	Dynamic scalability: cloud-native applications should be able to scale up or down dynamically depending on the load conditions.

### 4.4.4	Relevance of cloud-native design principles to 3GPP OAM

Table 4.4.4-1 provides a summary mapping between the "Twelve-factor app" [43] and "CNCF's cloud-native principles" [42]. Hence, where factors and principles are understood to be related, they are indicated on the same row in the table 4.4.4-1.

Table 4.4.4-1: Mapping between "Twelve-factor app" and "CNCF's cloud-native principles"

| Factor | Cloud-native principle |
| --- | --- |
| 1 – Codebase | Repeatable deployment process |
| 2 – Dependencies |  |
| 3 – Configuration |  |
| 4 – Backing services |  |
| 5 – Build, release, and run |  |
| 6 – Processes | Micro-services design Loosely coupled |
| 7 – Port binding |  |
| 8 – Concurrency | Dynamic scalability |
| 9 – Disposability | Resiliency |
| 10 – Dev/Prod parity |  |
| 11 – Logs | Observability |
| 12 – Administrative processes |  |
|  | Containerization |
|  | Immutable infrastructure |
|  | Declarative API |

The present document does not study and define the exact set of cloud native principles a CNF adheres to. It is not up to the 3GPP management system to mandate that network functions adhere to cloud-native principles in their design. Nevertheless, from a 3GPP management perspective, it is considered that a CNF can have the following aspects:

-	Highly scalable based on microservices.

-	Deployed using container technologies.

-	Multiple management interfaces to support multiple management applications.

The impact of the cloud-native design principles on the 3GPP management system needs to be investigated further.

## 4.5	Cloud deployment types

Management and orchestration solution for cloud deployments can support different types of cloud which the Communication Service Providers (CSPs) can use to deploy their solutions. Cloud deployments vary depending on the underling cloud environment based on ownership, scale, and access, as well as the cloud's nature and purpose. Typical cloud deployments in the industry include public-cloud, where the cloud infrastructure services are provided over the internet and available to the public, private-cloud, where the cloud services are accessible to one consumer, e.g. an organization and not reachable through internet, and hybrid-cloud, which is a combination of public and private clouds. Additionally, there are multi-cloud deployments, which consist of cloud infrastructure solutions provided by different cloud providers. It is important that CSPs are provided with support for various cloud deployment types and the desired level of control on managing the deployments.

# 5	Use cases, potential requirements, and potential solutions

## 5.1	Use of VNF generic OAM functions

### 5.1.1	Use case #1: Cloud-native VNF configuration management

#### 5.1.1.1	Description

Besides allocation and configuration of the necessary resources, to bring up a cloud native VNF into operation, i.e. full instance provisioning and configuration of the corresponding VNF instances needs to take place. Configuration actions can also be performed during the cloud native VNF instance operation.

An operator needs to be able to manage and orchestrate through the 3GPP management system the configuration of the cloud native VNF instances. The operator needs to be able to perform through the 3GPP management system, operations like set and query configuration information.

For data governance reasons, the operator needs to be able to control the location where configuration data is stored. Additional information is needed that can be used by the MnS producer to support operations like:

-	backup configuration information;

-	rollback configuration information (e.g., after reverting software to a previous stable version); and

-	remove unnecessary configuration files (e.g., very old versions).

#### 5.1.1.2	Potential requirements

REQ-CVNF_CM-1 The 3GPP management system should have the capability to perform configuration operations for cloud-native VNF instances.

REQ-CVNF_CM-2 The reference point between the 3GPP management system and external OAM entities should have the capability enabling the 3GPP management system to interact with external (non-3GPP) configuration management entities for the purpose of performing configuration operations for cloud-native VNF instances.

#### 5.1.1.3	Potential solutions

##### 5.1.1.3.1	VNF Configuration Manager function

This solution introduces a platform entity that interacts with the 3GPP management system via a new reference point for performing the configuration management of cloud-native VNFs.

This solution proposes the use of the VNF Configuration Manager function defined in ETSI GS NFV-IFA 049 [2]. Some key functionalities supported by the VNF Configuration Manager function are the capability to convey configuration information to one or more cloud-native VNF/VNFC instances, the capability to perform pre-configuration actions (e.g. create configuration backup) and post-configuration actions (e.g. rollback running configuration) and the capability to query configuration information of cloud-native VNF/VNFC instances.

From 3GPP management system perspective, MnS provisioning handles configuration of managed NFs defined by 3GPP, which include the necessary configuration related to the behaviour and role of a NF (e.g., configuration of a gNB CU UP function). The 3GPP management system fully understands the semantics of this kind of configuration since it is needed to ensure that the 3GPP mobile network operates as intended by the network operator.

The VNF Configuration Manager does not understand the semantics of the configuration information that is conveyed to the cloud-native VNF/VNFC instances (both VNF application configuration parameters and VNF non-application configuration parameters).

Figure 5.1.1.3.1-1 depicts the interaction and reference point between 3GPP management system and the VNF Configuration Manager.

Figure 5.1.1.3.1-1: Interaction and reference point between 3GPP management system and VNF Configuration Manager

The present solution addresses the potential requirements REQ-CVNF_CM-1 and REQ-CVNF_CM-2.

This potential solution related to option 2 in clause 5.1.5.

##### 5.1.1.3.2	Network Configuration Manager function

This solution introduces a platform entity that interacts with the 3GPP management system via a new reference point for performing the configuration management of cloud-native VNFs.

This solution considers the use of the Network Configuration Manager function defined in ETSI GS NFV-IFA 049 [2]. The Network Configuration Manager function can be used to set network configuration information related to one or more VNF/VNFC instances. For example, the Network Configuration Manager can be used to configure the VNF Connection Points (CPs), VNF networking for VNF instances in a service mesh, etc.

Figure 5.1.1.3.2-1 depicts the interaction and reference point between 3GPP management system and the Network Configuration Manager.

Figure 5.1.1.3.2-1: Interaction and reference point between 3GPP management system and Network Configuration Manager

The present solution addresses the potential requirements REQ-CVNF_CM-1 and REQ-CVNF_CM-2.

This potential solution related to option 2 in clause 5.1.5.

##### 5.1.1.3.3	Configuration Server

This solution introduces a platform entity that interacts with the 3GPP management system via a new reference point for performing the configuration management of cloud-native VNFs.

This solution proposes the use of a Configuration Server introduced in ETSI GR NFV-EVE 022 [44]. The Configuration Server can be understood as a logically centralized storage for configuration, without limiting the implementation of the storage. In addition, Configuration Server services can be used to store and fetch configuration data, convert configuration data formats, validate configuration data schema and apply version control for configuration data. The interface exposed and operations supported by the Configuration Server are defined in ETSI GS NFV-IFA 049 [2].

NOTE:	Implementation of the Configuration Server, e.g. structure of configuration data, format of storage is not in scope of the present solution.

Figure 5.1.1.3.3-1 depicts the interaction and reference point between 3GPP management system and the Configuration Server.

Figure 5.1.1.3.3-1: Interaction and reference point between 3GPP management system and Configuration Server

As depicted in figure 5.1.1.3.3-1, a consumer of the Configuration Server interfaces could be other VNF generic OAM function such as the VNF Configuration Manager defined in ETSI GS NFV-IFA 049 [2] or the 3GPP management system.

The solution enables the 3GPP management system having the capability to handle configuration data, like distributing, fetching and transforming configuration data, for one or multiple cloud-native VNFs.

The present solution addresses the potential requirements REQ-CVNF_CM-1 and REQ-CVNF_CM-2.

This potential solution related to option 2 in clause 5.1.5.

##### 5.1.1.3.4	Using the existing 3GPP provisioning management service and ETSI NFV MANO

In this solution a MnS producer offering the MnS provisioning service directly interacts with the VNF for VNF application specific parameters configuration purposes.

-	The 3GPP management system configures VNF with VNF application specific parameters by utilizing already defined 3GPP provisioning MnS (as defined in clause 11.1 of TS 28.532 [10]).

-	The 3GPP management system interacts with NFV-MANO to configure VNF non-application parameters (as defined in clause 5.1.18 of TS 28.531 [7]).

This solution is not using the VNF Generic OAM functions described in ETSI GS NFV-IFA 049 [2].

This potential solution related to option 1 in clause 5.1.5.

##### 5.1.1.3.5	Proposed updates in the 3GPP management system

From a 3GPP management system point of view, additional information is needed by the MnS producer to support:

-	Backup of configuration data.

-	Rollback of configuration data.

-	Removal of unnecessary configuration files (e.g., very old versions).

Editor's Note:	It is TBD to determine possible options to support the operations above (e.g., introduce a new parameter configBackupInfo under vnfParametersList defined in TS 28.622 [45] etc.).

##### 5.1.1.3.6	Using the existing 3GPP provisioning management service

In this solution a MnS producer offering the MnS provisioning service directly interacts with the VNF for both VNF application specific parameters configuration and VNF non-application parameters purposes.

-	The 3GPP management system configures the VNF with VNF application specific parameters by utilizing the already defined 3GPP provisioning MnS (as defined in clause 11.1 of TS 28.532 [10]) and the network resource model defined in TS 28.541 [17].

-	The 3GPP management system configures VNF with VNF non-application specific parameters by utilizing already defined 3GPP provisioning MnS (as defined in clause 11.1 of TS 28.532 [10]) and the VsDataContainer IOC defined in TS 28.622 [45]. The VNF non-application parameters (including both standardized and non-standardized parameters) can be carried by the VsDataContainer IOC.

This solution provides a unified solution for configuring both VNF application specific parameters configuration and VNF non-application specific parameters by utilizing the existing 3GPP provisioning management service. This mechanism also supports the association between VNF application specific parameters configuration and VNF non-application specific parameters configuration, for example, add VsDataContainer name contained by EP_RP to carry the CpConfiguration defined in ETSI GS NFV-IFA 008 [9] to support association between 3GPP defined EP_RP with ETSI NFV defined External Connection Point. Further, this solution supports carrying any type of configuration data that is required to support specific implementations.

The solution focusses on using 3GPP provisioning MnS for both VNF application and non-application configuration. How the MnS producer perform the configuration activities for the VNF is implementation specific. 3GPP Provisioning MnS don't understand semantics of the VNF non-application parameters.

This potential solution is related to option#3 in clause 5.1.5.

#### 5.1.1.4	Evaluation of solutions

The solutions presented in clauses 5.1.1.3.1, 5.1.1.3.2 and 5.1.1.3.3 describe how the VNF generic OAM functions can be used to convey/apply non-application configuration of cloud-native VNFs. The VNF generic OAM functions and corresponding interfaces are specified in ETSI GS NFV-IFA 049 [2]. Their specification is out of scope of 3GPP.

The solutions presented in clauses 5.1.1.3.4 and 5.1.1.3.6 relies on the use of existing SA5 specifications to support configuration of cloud-native VNFs. These solutions do not require any further enhancements to support the use case requirement REQ-CVNF_CM-1.

### 5.1.2	Use case #2：Cloud-native VNF policy management

#### 5.1.2.1	Description

3GPP TS 28.555 [12] specifies the concepts, requirements and use cases for network policy management in 5G networks. It is suitable for use cases where 3GPP and MANO interact for policy management (see the policy categories in ETSI GR NFV-IFA 023 [13] table 6.2.2-1 for details) and lacks support for the cloud native VNFs.

The 3GPP management system needs to implement policy management for cloud-native VNFs, which might be implemented by a Microservice-based architecture. This architecture can split a single application into multiple small services, each of them can run independently. However, it will also bring many challenges, such as a large number of services will have complex dependencies, resulting in complex deployment. In this case, policy management is needed to help improve efficiency.

#### 5.1.2.2	Potential requirements

REQ-policy-1 The 3GPP management system should have the capability to manage policies for the cloud-native VNFs.

REQ-policy-2 The reference point between 3GPP management system and external OAM entity should support the capability enabling the 3GPP management system to interact with external (non-3GPP) policy management entities to perform the policy management for cloud-native VNF.

#### 5.1.2.3	Potential solutions

##### 5.1.2.3.1	Policy Agent

As shown in figure 5.1.2.3.1-1, this solution introduces a platform entity that interacts with 3GPP management system for policy management of cloud-native VNFs via a new PaaS reference point.

This solution proposes using the Policy Agent function defined in ETSI GS NFV-IFA 049 [2], which can interact with the VNF generic OAM function, other PaaS Service, and VNFs for assisting on the execution and decision-making of policies.

Figure 5.1.2.3.1-1: Potential solution for Cloud-native VNF policy management using the Policy Agent

The solution improves the efficiency in handling policies associated to various entities by interacting with VNF generic OAM functions and other PaaS Services, and enables the 3GPP management system having the capability to manage policies for the cloud-native VNFs with or without interaction with NFV-MANO functions.

The present solution addresses the potential requirement REQ-policy-1 and REQ-policy-2.

This potential solution related to option 2 in clause 5.1.5.

##### 5.1.2.3.2	Use of existing 3GPP provisioning management service and ETSI NFV MANO

In this solution a MnS producer offering the MnS provisioning service directly interacts with the VNF for policy management purposes.

-	The 3GPP management system configures VNF application specific policies according to TS 28.556 [49] and by utilizing already defined 3GPP provisioning MnS (as defined in clause 11.1 of TS 28.532 [10]).

-	3GPP management system interacts with NFV-MANO to configure VNF non-application policies (as defined in clause 5.1.18 of TS 28.531 [7]).

This solution is not using the VNF Generic OAM functions described in ETSI GS NFV-IFA 049 [2].

This potential solution related to option 1 in clause 5.1.5.

#### 5.1.2.4	Evaluation of solutions

The solution presented in clause 5.1.2.3.1 describes how the VNF generic OAM functions can be used to convey/apply non-application policies related to the cloud-native VNF. The VNF generic OAM functions and corresponding interfaces are specified in ETSI GS NFV-IFA 049 [2]. Their specification is out of scope of 3GPP.

The solution presented in clause 5.1.2.3.2 relies on the use of existing 3GPP SA5 specifications to support policy management for the cloud-native VNFs. The solution does not require any further enhancements in the 3GPP management system to satisfy the use case requirement REQ-policy-1.

### 5.1.3	Use case #3: Cloud-native VNF Traffic management

#### 5.1.3.1	Description

Effective traffic management for cloud-native functions is essential to ensure high quality of service levels. Nevertheless, especially in containerized deployments additional challenges need to be considered since many OS containers realizing the cloud-native VNF instances are usually deployed per host, while OS containers are often created and destroyed rapidly, requiring the network to adapt quickly to topological changes. In the context of a cloud-native VNF, traffic management includes controlling the inbound/outbound traffic to, from and within the cloud-native VNF instance.

Traffic management actions to be considered in a 3GPP management system context are, for example, controlling the rate of incoming requests to prevent overloading services and directing traffic to different VNFCs realizing a cloud-native VNF. These actions can surge in the context of diverse OAM procedures of maintenance, re-configuration, and upgrade of NFs, etc. managed through the 3GPP management system.

The 3GPP management system needs to be able to support an operator to manage and orchestrate the traffic management actions for cloud-native VNF instances.

#### 5.1.3.2	Potential requirements

REQ-CVNF_TM-1 The 3GPP management system should have the capability to support traffic management of cloud-native VNF instances.

REQ-CVNF_TM-2 The reference point between 3GPP management system and external OAM entity should have the capability enabling the 3GPP management system to interact with external (non-3GPP) traffic management entities for the purpose of performing traffic management for cloud-native VNF instances.

#### 5.1.3.3	Potential solutions

##### 5.1.3.3.1	Traffic Enforcer function

As shown in figure 5.1.3.3.1-1, this solution introduces a platform entity that interacts with 3GPP management system for traffic management of cloud-native VNFs via a new PaaS reference point.

Figure 5.1.3.3.1-1: Traffic management of cloud native VNF

The solution proposes using Traffic Enforcer function defined in ETSI GS NFV-IFA 049 [2], which is one of the VNF generic OAM functions. Some key functionalities supported by the Traffic Enforcer function are the capability to perform the required traffic blocking and rerouting operations on the VNFC instances.

According to ETSI GS NFV-IFA 049 [2] Traffic Enforcer functionality can be called by functions residing inside the 3GPP management system or other VNF generic OAM functions (e.g. the Upgrade VNF function) or other PaaS Services (e.g. the Policy Agent).

The present solution addresses the potential requirement REQ-CVNF_TM-2.

#### 5.1.3.4	Evaluation of solutions

The role of the 3GPP management system in supporting traffic management (as per the use case requirement REQ-CVNF_TM-1) is ambiguous. The use case has not defined what "Cloud-native VNF traffic" means and the relation if any with 3GPP signalling traffic.

Subsequently, if cloud-native VNF traffic is not the 3GPP signalling traffic, then the traffic management use case is out of scope for 3GPP SA5 in this release.

### 5.1.4	Use case #4: Cloud-native VNF Upgrade

#### 5.1.4.1	Description

VNF upgrades typically address known bugs and issues, leading to more stable and reliable operations. In addition, VNF upgrades can include new features and functionalities and/or security patches that fix vulnerabilities. Cloud native VNF upgrades ensure that VNFs remain compatible with the latest standards specification facilitating the seamless integration with existing and new VNFs and management systems.

In the industry different VNF upgrade strategies are met in practice such as blue-green updates (traffic is served by a part of the network using the old version, while another part of the network is updated and tested with the new version) and canary updates (small subset of systems is updated and is also used to serve part of the traffic, before updating the entire production environment). In both cases, proper setup and configuration is needed, such as transferring of images, transferring of configuration files, configuring the load balancers, etc.

Both virtualization dependent (e.g. VNF network configuration, virtual or physical resource management) and virtualization independent aspects (e.g. NF related) can be considered during a cloud-native VNF upgrade.

When cloud-native VNF are considered, upgrades become challenging due to the multiple components, software artifacts and configuration files to be handled.

An operator needs to be able to manage and orchestrate through the 3GPP management system the upgrade of the cloud native VNF instances. The operator needs to be able to perform through the 3GPP management system, operations like set and query upgrade files information.

#### 5.1.4.2	Potential requirements

REQ-CVNF_UP-1 The 3GPP management system should have the capability to upgrade cloud-native VNF instances.

REQ-CVNF_UP-2 The reference point between 3GPP management system and external OAM entity should have the capability enabling the 3GPP management system to interact with external (non-3GPP) upgrade management entities for the purpose of upgrading cloud-native VNF instances.

#### 5.1.4.3	Potential solutions

##### 5.1.4.3.1	Upgrade VNF function

This solution introduces a platform entity that interacts with the 3GPP management system via a new reference point for performing the upgrade of cloud-native VNFs.

This solution proposes the use of the Upgrade VNF function defined in ETSI GS NFV-IFA049 [2]. Some key functionalities supported by the Upgrade VNF function are, for example, the capability to modify the software of a VNF to another version and the capability to add resources to a running VNFC instance during an upgrade by coordinating with VNF management function(s) responsible for managing the resources of the VNF instance.

The VNF upgrade can be supported by the change VNF package procedures, which are followed based on the information available in the cloud-native VNF descriptor and/or updated cloud-native VNF Package files. The Upgrade VNF function can be also used to orchestrate an upgrade for multiple cloud-native VNFs/VNFCs. It can also manage both virtualization dependent and virtualization independent aspects.

To perform the intended functionality, the Upgrade VNF function can interact with other entities, as illustrated in the following examples.

EXAMPLE 1:	In the case of ETSI NFV based solutions, the Upgrade VNF function can coordinate with the VNFM, which executes the lifecycle according to the NFV-MANO procedures including handling the removal/addition/modification of resources.

EXAMPLE 2:	To update the network configuration of the connection points of a VNFC, the Upgrade VNF function can interact with the Network Configuration Manager defined in ETSI GS NFV-IFA 049 [2].

A consumer of the Upgrade VNF function interfaces can be the 3GPP management system over a reference point between the 3GPP management system and the platform providing VNF generic OAM and other PaaS Services. This is depicted in figure 5.1.4.3.1-1.

Figure 5.1.4.3.1-1: Interaction and reference point between 3GPP management system and Upgrade VNF function

The solution enables the 3GPP management system having the capability to manage the upgrade of cloud-native VNFs by interacting with the Upgrade VNF function, who takes the responsibility of handling the various components, artifacts and configurations files and the necessary interactions with the cloud related management and orchestration system.

The present solution addresses the potential requirements REQ-CVNF_UP-1 and REQ-CVNF_UP-2.

This potential solution related to option 2 in clause 5.1.5.

##### 5.1.4.3.2	Use of existing 3GPP provisioning management service and ETSI NFV MANO

In this solution a MnS producer offering the MnS provisioning service directly interacts with the VNF for VNF upgrade purposes. The 3GPP management system upgrade VNF with VNF application specific items by utilizing the already defined 3GPP provisioning MnS (as defined in clause 11.1 of TS 28.532 [10]).

3GPP management system interacts with NFV-MANO to upgrade VNF non-application items (as defined in clause 5.1.18 of TS 28.531 [7]).

This solution is not using the VNF Generic OAM functions described in ETSI GS NFV-IFA 049 [2].

This potential solution related to option 1 in clause 5.1.5.

#### 5.1.4.4	Evaluation of solutions

The solution presented in clause 5.1.4.3.1 describes how the VNF generic OAM functions can be used to support the upgrade of the non-application parameters of the cloud-native VNFs. The VNF generic OAM functions and corresponding interfaces is specified in ETSI GS NFV-IFA 049 [2]. Their specification is out of scope of 3GPP.

The solution presented in clause 5.1.4.3.2 relies on the use of existing SA5 specifications to support the upgrade of cloud-native VNFs. The solution does not require any further enhancements in the 3GPP management system to satisfy the use case requirement REQ-CVNF_UP-1.

### 5.1.5	Relationship between 3GPP Management System (SBMA) and ETSI NFV VNF generic OAM functions and NFV-MANO

The decoupling of software and hardware in NFV gave rise to the need to delineate the concerns of application-level management versus virtualization-related management. VNF application specific parameters are related to the behaviour and services provided by the NF, which in the 3GPP context are therefore 3GPP service related and defined by 3GPP, whilst the VNF non-application specific parameters (i.e. non-3GPP service related) are related to the virtualization aspects of the NF and consider the aspects on how software entities can be deployed and operated on virtualized infrastructure platform. Following are the details:

-	The MO attributes, which are defined by 3GPP, can be regarded to be VNF application specific parameters.

-	The VNF instance specific information defined in ETSI NFV specifications can be regarded to be VNF non-application specific parameters.

NOTE 1:	The VNF application specific parameters are referred as virtualization-independent items in ETSI IFA 049 [2], VNF non-application specific parameters are referred as virtualization dependent items in ETSI IFA 049 [2].

Regarding the relationship between the VNF generic OAM functions framework and 3GPP management system (SBMA) defined in TS 28.533 [16] the following can be considered:

-	Like in the case of NFV-MANO, VNF generic OAM functions reside outside the 3GPP management system (SBMA).

-	In SBMA, a MnS producer can consume management interfaces provided by NFV-MANO and can expose a corresponding service to MnS consumers. Similarly for the case of VNF Generic OAM functions in SBMA, a MnS producer can consume management interfaces provided by VNF generic OAM functions and can expose a corresponding service to MnS consumers.

-	Following are the two options to illustrate the relation between 3GPP management system (SBMA),VNF generic OAM functions and NFV-MANO.

NOTE 2:	The name of VNF application parameters and VNF non-application parameters needs to be revisited during normative phase.

Option#1:

Figure 5.1.5-1: Use of 3GPP MnS and ETSI NFV MANO to manage VNF

In this option, 3GPP management system (SBMA) configures/manages NF realized by VNF(s) with VNF application specific parameters by utilizing already defined 3GPP MnSs (e.g. provisioning MnS). 3GPP management system interacts NFV-MANO to configure/manage NF realized by VNF(s) with VNF non-application specific configurations.

Option#2:

Figure 5.1.5-2: Interactions between SBMA and VNF Generic OAM Functions

In this option, 3GPP management system (SBMA) still manages VNF non-application specific parameters with NFV-MANO by utilizing already defined 3GPP MnSs. In addition the 3GPP management system interacts with PaaS Services defined in ETSI ISG NFV-IFA 049 [2] (i.e., VNF Generic OAM functions) to manage VNF non-application specific aspects (typically those which are not managed by NFV-MANO or resulting from NFV-related orchestration procedures) but also VNF application specific aspects (e.g., VNF configuration backup etc.).

In the case of VNF application specific aspects, VNF generic OAM functions, which do not process the semantics of application specific parameters, serving simply as utility functions in the cloud-native platform with specific OAM capabilities.

Option#3:

Figure 5.1.5-3: Use of 3GPP MnS as unified solution

In this option, 3GPP management system (SBMA) configures/manages NF realized by VNF(s) with VNF application specific parameters and VNF non-application specific parameters (typically those which are not managed by NFV-MANO or resulting from NFV-related orchestration procedures) by utilizing already defined 3GPP MnSs (e.g. provisioning MnS). The VNF non-application specific parameters can be carried within VsDataContainer defined in TS 28.622 [45]. The VNF application specific parameters and VNF non-application specific parameters can be specified by MnS consumer or derived from MnS Producer.

For VNF non-application parameters which standardized by ETSI NFV, 3GPP Management System can use 3GPP MnSs with VsDataContainer to configure VNF or interact with NFV-MANO (see the "dotted-line") to configure VNF.

In the case of VNF non-application specific aspects, the 3GPP MnSs don't understand the semantics of VNF non-application specific parameters. This solution serves simply to provide unified OAM capabilities for managing NF realized by VNF(s).

## 5.2	Use of industry solutions for management of NF Deployments

### 5.2.1	Use case #1: 3GPP management architecture evolution to support LCM of NF Deployment instance

#### 5.2.1.1	Description

3GPP specifications specify the interaction with ETSI NFV MANO for the LCM of the virtualized part of NF (VNF). The requirements, use cases and procedures for the management of NF containing the virtualized part are specified in TS 28.531 [7] for 5G network. The interfaces for the interaction with ETSI NFV MANO are provided via Os-Ma-nfvo and the Ve-Vnfm-em reference points specified in ETSI GS NFV-IFA 013 [8] and ETSI GS NFV-IFA 008 [9].

There are newly developed and evolved industry solutions for management and orchestration of cloud native applications that leverage industry standards, e.g. Kubernetes based solution. Such industry solutions including ETSI NFV MANO and non ETSI NFV MANO can be used to address challenges related to NF Deployment instances e.g. for hybrid cloud deployments that deploy cloud native applications with hyperscale cloud providers. This use case considers the scenarios where the 3GPP management architecture is flexible to support use of ETSI NFV MANO and Non ETSI NFV MANO for the LCM of NF Deployment instance.

#### 5.2.1.2	Potential requirements

REQ_LCM-1 The deployment management reference point should support the capability allowing the 3GPP management system to interact with an orchestration and management entity to perform the LCM of NF Deployment instance.

#### 5.2.1.3	Potential solutions

Introduce a generic orchestration and management entity that interacts with 3GPP management system for LCM of NF Deployment instance via the new deployment management reference point.

The figure 5.2.1.3-1 below illustrates the interaction between 3GPP management system and orchestration and management entity in simplified view:

Figure 5.2.1.3-1: Lifecycle management of NF Deployment instance

The deployment management reference point includes the interfaces that 3GPP supports today with ETSI NFV MANO but not limited to it, e.g. Kubernetes based API. For the case of NFV-MANO the deployment management reference point includes the exchange of information over the interfaces that 3GPP supports with ETSI NFV MANO (ETSI GS NFV-IFA 013 [8] and ETSI GS NFV-IFA 008 [9]).

#### 5.2.1.4	Evaluation of solutions

The solution in clause 5.2.1.3 describes use of a deployment management reference point that enables interaction of 3GPP management system with an orchestration and management system.

The proposed reference point fulfils the use case requirement expressed in clause 5.2.1.2.

### 5.2.2	Use case #2: data streaming for cloud-native NF deployment

#### 5.2.2.1	Description

Currently 3GPP management system support WebSocket based data streaming for PM, tracing and analytic TS 28.532 [10] which establish point-to-point connection between the streaming data reporting MnS consumer and MnS producer. In cloud native deployment, a NF is realized in many micro-services whose workload instances are running in parallel, dynamically scaled in and out, and maybe distributed across multiple server nodes and cloud sites. In conventional WebSocket based solution, streaming data reporting MnS producer aggregate traffic from many distributed workloads generating large amount of data before streaming it to the streaming data reporting MnS consumer via the connection established. The streaming data reporting MnS consumer receive the aggregated traffic first via the connection before distributing to internal functions of the management system which may be also cloud native. This become a performance bottleneck, inefficient and difficult to be managed. Furthermore, cloud native applications are more sensitive to failures and system down time. If the connection between MnS producer and consumer fails due to any reason e.g. software failure, server hardware failure and transport network failure etc, it impacts the entire coverage area of the cloud.

There are other industry solutions which are based on stateless and multipoint-to-multipoint communications (e.g. NF micro-service workloads to 3GPP management system microservice workloads) widely used in large-scale cloud environment. They were designed natively to tackle the challenges above, improving scalability, efficiency, resiliency and cost. The 3GPP management system should evolve to address the challenges considering the use of those existing industry solutions.

#### 5.2.2.2	Potential requirements

PREQ-FS_Cloud-OAM-01 The 3GPP management system should have the capability enabling the MnS consumer to configure the MnS producer for the streaming of management data based on message bus.

NOTE:	The management data includes performance measurements or KPIs defined in TS 28.552 [14] and TS 28.554 [15], trace reporting data and analytic reporting data TS 28.532 [10].

PREQ-FS_Cloud-OAM-02 The 3GPP management system should have the capability enabling the MnS producer to stream management data to the MnS consumer based on message bus.

#### 5.2.2.3	Potential solutions

##### 5.2.2.3.1	Solution #1: Management data streaming based on message bus

This potential solution proposes the addition of a new reporting method for management data for network functions running in the cloud. The solution proposes the use of message bus for the reporting of management data from the MnS producer to the MnS consumer as shown in figure 5.2.2.3.1-1.

Figure 5.2.2.3.1-1: Potential solution for management data streaming based on message bus

The proposed solution enables MnS producer(s) to stream management data to the message bus when ready while also enabling the MnS consumer(s) to consume the data from the message bus whenever they need to. In addition, this potential solution decouples the production of management data from the consumption of management data.

##### 5.2.2.3.2	Solution #2: Management data streaming for cloud-native NF deployments

The proposed solution relies on existing streaming mechanisms to stream management data between the MnS producer and MnS consumer (as defined in clause 12.5 of TS 28.532 [10]) as shown in figure 5.2.2.3.2-1.

The proposed solution supports cloud-native deployments on the MnS producer and MnS consumer sides which comes with the benefits of scalability, redundancy and fault-tolerance. If the MnS producer(s) leverage micro-service-based stateless architectures typical of cloud-native deployments, then it's implementation specific how the MnS producers keep track of the stateful nature of the web-socket connection.

Figure 5.2.2.3.2-1: Management data streaming for cloud-native NF deployments

#### 5.2.2.4	Evaluation of solutions

Solution #1 in clause 5.2.2.3.1 provides data streaming based on message bus. The proposed solution implies impacts on the 3GPP management system to support the streaming of management data based on message bus. This includes:

1	Impact to existing streaming data reporting service defined in TS 28.532 [10]: to support message bus-based data reporting from the MnS producer, a new streaming data reporting service needs to be introduced.

2	Impact to MnS producer due to the generality of data streaming service.

Solution #2 in clause 5.2.2.3.2 shows how management data can be streamed with minimal impact to the existing MnS producers. The proposed solution relies on existing SA5 defined mechanisms for streaming management data between the MnS producer and the MnS consumer. It implies no impacts on the 3GPP management system.

### 5.2.3	Use case #3: Creation of a NF Deployment instance

#### 5.2.3.1	Description

In this use case the 3GPP management system interacts with an orchestration and management entity for creation of a NF Deployment instance.

In order to deploy a NF Deployment instance the 3GPP management system conveys the description of the NF Deployment requirements and parameters to the orchestration and management entity. The collection of this information, referred to as descriptor, includes all necessary information needed by the orchestration and management entity to select and configure the appropriate cloud resources and to deploy the NF Deployment instance.

The 3GPP management system needs to be able to request an orchestration and management entity to create a NF Deployment instance.

#### 5.2.3.2	Potential requirements

REQ_CMO-LCM-CREATE-1 The deployment management reference point should have the capability enabling the 3GPP management system to request creation of a NF Deployment instance based on NF Deployment requirements.

REQ-CMO-LCM-CREATE-2 The deployment management reference point should have the capability enabling the orchestration and management entity to inform the 3GPP management system of the result of the request for creation of a NF Deployment instance.

#### 5.2.3.3	Potential solutions

##### 5.2.3.3.1	Use of deployment management reference point

In this solution the 3GPP management system interacts with an orchestration and management entity using the deployment management reference point as described in clause 5.2.1.3 for creation of a NF Deployment instance.

Figure 5.2.3.3.1-1 depicts a high-level view of proposed procedure for creation of a NF Deployment instance.

Figure 5.2.3.3.1-1: High level procedure for NF Deployment instance creation procedure

The 3GPP management system requests creation of a NF Deployment instance over deployment management reference point, and receives the result of the request for creation of the NF Deployment instance from the orchestration and management entity. If the orchestration and management entity is ETSI NFV MANO, the interactions over deployment management reference point are as specified in clause 7.10 of TS 28.531 [7]. If another orchestration and management entity is considered, updates are needed in TS 28.531 [7] and possibly other specifications to describe the interactions over the deployment management reference point.

##### 5.2.3.3.2	Use of deployment management reference point based on declarative descriptor

In this solution the 3GPP management system interacts with an orchestration and management system using the deployment management reference point as described in clause 5.2.1.3 for creation of a NF Deployment instance. The deployment requirements for creating NF Deployment instance(s) are conveyed from the 3GPP management system to the orchestration and management system via a declarative descriptor.

Figure 5.2.3.3.2-1 depicts a high-level view of proposed procedure for creation of a NF Deployment instance based on declarative descriptor.

Figure 5.2.3.3.2-1: Interaction between 3GPP management system and orchestration and management system using deployment management reference point based on declarative descriptor

The declarative descriptor provides a declaration in high-level on what to be achieved by the orchestration and management system rather than how to achieve it.

If the orchestration and management system is ETSI NFV MANO, the interactions over deployment management reference point are as specified in clause 7.10 of TS 28.531 [7]. For the case of NFV-MANO, the declarative descriptor is VNFD, as per ETSI NFV specifications (see ETSI GS NFV-IFA 011 [22]). If another orchestration and management system is considered, updates are needed in TS 28.531 [7] and possibly other specifications to describe the interactions over the deployment management reference point and to define the declarative descriptor.

NOTE:	In case the orchestration and management system is ETSI NFV MANO, VNFD is defined to convey the deployment requirement information. The ETSI GS NFV-IFA 007 [19] defines the Instantiate VNF interface for NF Deployment creation use case. For other industry solutions, currently there is no standardized descriptor.

#### 5.2.3.4	Evaluation of solutions

The proposed solutions in clause 5.2.3.3 describe the high level interactions for use of the reference point introduced in clause 5.2.1.3 for creation of NF Deployment(s).

The proposed solution in clause 5.2.3.3.2 describes use of declarative descriptor for NF Deployment creation.

In the proposed solution the non-ETSI MANO interfaces are unspecified because they are out of scope of 3GPP, and the completion of the creation operation is dependent on the capabilities provided by the external orchestration and management system.

With the interfaces provided by an external orchestration and management system that has the capability to perform creation of NF Deployment, the 3GPP management system is able to send the request and receive the response for the creation of NF Deployment, and such solutions can fulfil the requirements expressed in clause 5.2.3.2.

### 5.2.4	Use case #4: Modification of a NF Deployment instance

#### 5.2.4.1	Description

In this use case the 3GPP management system interacts with an orchestration and management system for modification of a NF Deployment instance.

An already deployed NF Deployment instance may be modified. In order to modify a NF Deployment instance an updated description of the NF Deployment requirements and parameters can be conveyed to the orchestration and management system.

The 3GPP management system needs to be able to request an orchestration and management system to modify a NF Deployment instance.

#### 5.2.4.2	Potential requirements

REQ-LCM-MODIFY-1: The deployment management reference point should have the capability enabling the 3GPP management system to request modification of a NF Deployment instance based on NF Deployment specific requirements.

REQ-LCM-MODIFY-2: The deployment management reference point should have the capability enabling the orchestration and management system to inform the 3GPP management system of the result of the request for modification of a NF Deployment instance.

#### 5.2.4.3	Potential solutions

##### 5.2.4.3.1	Use of deployment management reference point

In this solution the 3GPP management system interacts with an orchestration and management system using the deployment management reference point as described in clause 5.2.1.3, for modification of a NF Deployment instance.

Figure 5.2.4.3.1-1 depicts a high-level view of proposed procedure for modification of a NF Deployment instance.

Figure 5.2.4.3.1-1: High level procedure for NF Deployment instance modification procedure

The 3GPP management system requests modification of a NF Deployment instance over deployment management reference point, and receives the result of the request for modification of the NF Deployment instance from the orchestration and management system.

If the orchestration and management system is ETSI NFV MANO, the interactions over deployment management reference point are as specified in clause 7.11 of TS 28.531 [7]. If another orchestration and management entity is considered, updates are needed in TS 28.531 [7] and possibly other specifications to describe the interactions over the deployment management reference point.

##### 5.2.4.3.2	Use of deployment management reference point based on declarative descriptor

In the cloud industry, declarative descriptors are used for management. The below section provides one example of using declarative descriptor for modification of NF Deployment instance(s).

In this solution, the 3GPP management system interacts with an orchestration and management system using the deployment management reference point as described in clause 5.2.1.3 for modification of NF Deployment instance(s). The deployment requirements for modifying NF Deployment instance(s) are conveyed from the 3GPP management system to the orchestration and management system via an updated version of the declarative descriptor that was used for creation of the NF Deployment instance.

Figure 5.2.4.3.2-1 depicts a high-level view of proposed procedure for modification of a NF Deployment instance based on declarative descriptor.

Figure 5.2.4.3.2-1: Interaction between 3GPP management system and orchestration and management system using deployment management reference point based on declarative descriptor

For the case of NFV-MANO, the declarative descriptor is an updated VNFD as per ETSI NFV specifications (see ETSI GS NFV-IFA 011 [22]).

NOTE:	In case the orchestration and management system is ETSI NFV MANO, an updated VNFD can be used to convey the modified deployment requirement information. The ETSI GS NFV-IFA 007 [19] defines the interface (Change VNF package operation) for the VNF lifecycle management operations. For other industry solutions, currently there is no standardized descriptor.

#### 5.2.4.4	Evaluation of solutions

The proposed solutions in clause 5.2.4.3 describe the high level interactions for use of the reference point introduced in clause 5.2.1.3 for modification of NF Deployment(s).

The proposed solution in clause 5.2.4.3.2 describes use of declarative descriptor for NF Deployment modification.

In the proposed solution the non-ETSI MANO interfaces are unspecified because they are out of scope of 3GPP, and the completion of the modification operation is dependent on the capabilities provided by the external orchestration and management system.

With the interfaces provided by an external orchestration and management system that has the capability to perform modification of NF Deployment, the 3GPP management system is able to send the request and receive the response for the modification of NF Deployment, and such solutions can fulfil the requirements expressed in clause 5.2.4.2.

### 5.2.5	Use case #5: Termination of a NF deployment instance

#### 5.2.5.1	Description

In this use case the 3GPP management system interacts with an orchestration and management system for termination of a NF deployment instance.

A deployed NF deployment instance can be terminated by the 3GPP management system. To terminate a NF deployment instance there is the need to request the termination of the NF deployment instance from the orchestration and management system.

The 3GPP management system needs to be able to request an orchestration and management system to terminate a NF deployment instance.

#### 5.2.5.2	Potential requirements

REQ-LCM-TERMINATE-1 The deployment management reference point should have the capability enabling the 3GPP management system to request termination of a NF deployment instance.

REQ-LCM-TERMINATE-2 The deployment management reference point should have the capability enabling the orchestration and management system to inform the 3GPP management system of the result of the request for termination of a NF deployment instance.

#### 5.2.5.3	Potential solutions

##### 5.2.5.3.1	Use of deployment management reference point

In this solution the 3GPP management system interacts with an orchestration and management system using the deployment management reference point as described in clause 5.2.1.3, for termination of a NF deployment instance.

Figure 5.2.5.3.1-1 depicts a high-level view of proposed procedure for termination of a NF deployment instance.

Figure 5.2.5.3.1-1: High level procedure for NF deployment instance termination

The 3GPP management system requests termination of a NF deployment instance from an orchestration and management system for LCM of NF deployment instances over deployment management reference point, and receives the result of the request for termination of the NF deployment instance from the orchestration and management system.

Annex F provides an example scenario using interactions with orchestration and management system for termination of NF deployment instances. If the orchestration and management system is ETSI NFV MANO, the interactions over deployment management reference point are as specified in clause 7.12 of TS 28.531 [7]. If another orchestration and management entity is considered, updates are needed in TS 28.531 [7] and possibly other specifications to describe the interactions over the deployment management reference point.

##### 5.2.5.3.2	Use of deployment management reference point based on declarative descriptor

In the cloud industry, declarative descriptors are used for management. The below section provides one example of using declarative descriptor for termination of NF Deployment instance(s).

In this solution, the 3GPP management system interacts with an orchestration and management system using the deployment management reference point as described in clause 5.2.1.3 for termination of NF Deployment instance(s).

Figure 5.2.5.3.2-1 depicts a high-level view of proposed procedure for termination of a NF Deployment instance based on declarative descriptor.

Figure 5.2.5.3.2-1: Interaction between 3GPP management system and orchestration and management system using deployment management reference point based on declarative descriptor

For the case of NFV-MANO, if VNFD was used to instantiate a NF Deployment instance, it can be terminated using the 'Terminate VNF' operation as defined in ETSI GS NFV-IFA 007 [19].

The other option is to delete the same declarative descriptor used during the creation of NF Deployment(s) over the deployment management reference point.

#### 5.2.5.4	Evaluation of solutions

The proposed solutions in clause 5.2.5.3 describe the high level interactions for use of the reference point introduced in clause 5.2.1.3 for termination of NF Deployment(s).

The proposed solution in clause 5.2.5.3.2 describes use of declarative descriptor for NF Deployment termination.

In the proposed solution the non-ETSI MANO interfaces are unspecified because they are out of scope of 3GPP, and the completion of the termination operation is dependent on the capabilities provided by the external orchestration and management system.

With the interfaces provided by an external orchestration and management system that has the capability to perform termination of NF Deployment, the 3GPP management system is able to send the request and receive the response for the termination of NF Deployment, and such solutions can fulfil the requirements expressed in clause 5.2.5.2.

### 5.2.6	Use case #6: Scaling of NF Deployment instance(s)

#### 5.2.6.1	Description

There can be scenarios where the 3GPP management system needs to interact with the orchestration and management entity to request the horizontal scaling of a specific NF Deployment instance. The horizontal scaling determination can be based on, for example, the current or predicted load of the network function(s).

Also, the 3GPP management system can require to configure the scaling information on the orchestration and management entity to automatically trigger the horizontal scaling for specific NF Deployment instance. And depending on the configured scaling information by the 3GPP management system, the orchestration and management entity can automatically trigger the horizontal scaling of the NF Deployment instance(s).

#### 5.2.6.2	Potential requirements

REQ-SCALING-1 The deployment management reference point should have the capability enabling the 3GPP management system to request the horizontal scaling of the NF Deployment instance(s).

REQ-SCALING-2 The deployment management reference point should have the capability enabling the orchestration and management entity to inform the 3GPP management system of the result of the horizontal scaling of specific NF Deployment instance request.

REQ-SCALING-3 The deployment management reference point should have the capability enabling the 3GPP management system to configure the scaling information to support the automatic horizontal scaling of specific NF Deployment instance.

REQ-SCALING-4 The deployment management reference point should have the capability enabling the orchestration and management entity to inform the 3GPP management system of the result of the configuration of the scaling information to support the automatic horizontal scaling of specific NF Deployment instance.

#### 5.2.6.3	Potential solutions

##### 5.2.6.3.1	Use of deployment management reference point

In the proposed solution, the 3GPP management system interacts with the orchestration and management entity to request the horizontal scaling of a specific NF Deployment instance.

To request the horizontal scaling of a specific NF Deployment instance, the 3GPP management system interact with the orchestration and management entity to request the horizontal scaling of specific NF Deployment instance. For the horizontal scaling request, the 3GPP management system can specify, for example, the component instances of the NF Deployment instance(s) or NF Deployment instance(s) to be scaled and the maximum number of scaled instances.

For the configuration of the scaling information for the orchestration and management entity to automatically trigger the horizontal scaling of the NF Deployment instance, the 3GPP management system can specify the scaling information. The scaling information can specify for example: the targeted NF Deployment instance, the activation/de-activation of autoscaling, the horizontal scaling triggering metrics or conditions (e.g. CPU usage level, memory usage level, and any other custom metrics), the minimum and maximum number of horizontally scaled instances, the cool-down period (i.e. waiting time before further horizontally scaling the NF Deployment instance) and the sync period (i.e. how often to check the defined triggering metrics).

These high-level interactions are shown in figure 5.2.6.3.1-1.

Figure 5.2.6.3.1-1: High-level interactions between the 3GPP management system and the orchestration and management entity to scale NF Deployment instance(s)

If the orchestration and management entity is ETSI NFV MANO, the interactions over deployment management reference point are as specified in clause 7.11 of TS 28.531 [7]. If another orchestration and management entity is considered, updates are needed in TS 28.531 [7] and possibly other specifications to describe the interactions over the deployment management reference point.

##### 5.2.6.3.2	Use of deployment management reference point based on declarative descriptor

In the cloud industry, declarative descriptors are used for management. The below section provides one example of using declarative descriptor for scaling of NF Deployment instance(s).

In this solution, the 3GPP management system interacts with an orchestration and management system using the deployment management reference point as described in clause 5.2.1.3 for scaling of NF Deployment instance(s). The deployment requirements for scaling NF Deployment instance(s) are conveyed from the 3GPP management system to the orchestration and management system via an updated version of the declarative descriptor that was used for creation of the NF Deployment instance.

Figure 5.2.6.3.2-1 depicts a high-level view of proposed procedure for scaling of a NF Deployment instance based on declarative descriptor.

Figure 5.2.6.3.2-1: Interaction between 3GPP management system and orchestration and management system using deployment management reference point based on declarative descriptor

For the case of NFV-MANO, the declarative descriptor is an updated VNFD as per ETSI NFV specifications (see ETSI GS NFV-IFA 011 [22]).

NOTE:	In case the orchestration and management system is ETSI NFV MANO, an updated VNFD can be used to convey the scaled deployment requirement information. The ETSI GS NFV-IFA 007 [19] defines the interface (Scale VNF or Scale VNF to Level operation) for the VNF lifecycle management operations. For other industry solutions, currently there is no standardized descriptor.

#### 5.2.6.4	Evaluation of solutions

The proposed solutions in clause 5.2.6.3 describe the high level interactions for use of the reference point introduced in clause 5.2.1.3 for scaling of NF Deployment(s).

The proposed solution in clause 5.2.6.3.2 describes use of declarative descriptor for NF Deployment scaling.

In the proposed solution the non-ETSI MANO interfaces are unspecified because they are out of scope of 3GPP, and the completion of the scaling operation is dependent on the capabilities provided by the external orchestration and management system.

With the interfaces provided by an external orchestration and management system that has the capability to perform scaling of NF Deployment, the 3GPP management system is able to request the horizontal scaling of specific NF Deployment instance(s) and such solutions can fulfil the requirements expressed in clause 5.2.6.2.

### 5.2.7	Use case #7: Observability for Network Functions in supporting cloud native

#### 5.2.7.1	Description

Current network management solutions have disaggregated means of transferring data associated with metrics, events, logs and traces in term of protocols and data format.

The management of NFs from multiple vendors should be simplified through a standardized and unified 3GPP management system with open interfaces and enabling a simplified operational model for the operators. As we move to a native cloud architecture, it is essential that standards related to network management ensure efficiency and increased service resiliency.

The 3GPP management system needs to consider the collection of metrics, events, logs and traces management data for network functions in supporting cloud native.

The following elements represent principles behind a well-defined observability solution:

-	Observability: The 3GPP management system needs to support robust observability mechanisms for proactive telemetry data analysis, failure detection and rapid response.

-	Interoperability: The 3GPP management system needs to ensure interoperability between different vendors, providers and operators for harmonized network functions management in a multi-cloud environment.

-	Energy Efficiency: The 3GPP management system needs to consider Energy Efficiency by design from a network management perspective.

-	Security: The 3GPP management system needs to implement security by design for open observability protecting confidentiality and integrity of data transfer.

-	AI driven: The 3GPP management system needs to support the adoption of AI-driven approaches to enhance observability data in order to understand the root-cause analysis, data transaction monitoring, automated discovery, geo-hosted network functions topology mapping, predictive maintenance, capacity planning, security threats identification and utilization for further optimization (as examples and not necessarily limited to).

#### 5.2.7.2	Potential requirements

PREQ-OBS_CNF-Cloud-OAM-01: The 3GPP management system should have the capability to generate Metrics to the MnS consumer.

PREQ-OBS_CNF-Cloud-OAM-02: The 3GPP management system should have the capability to generate Logs to the MnS consumer.

PREQ-OBS_CNF-Cloud-OAM-03: The 3GPP management system should have the capability to generate Traces to the MnS consumer.

PREQ-OBS_CNF-Cloud-OAM-04: The 3GPP management system should have the capability to support standard data format.

PREQ-OBS_CNF-Cloud-OAM-05: The 3GPP management system should have the capability to support standard data collection and reporting mechanisms.

PREQ-OBS_CNF-Cloud-OAM-06: The 3GPP management system should have the capability to provide telemetry data correlation capabilities between the network functions.

PREQ-OBS_CNF-Cloud-OAM-07: The 3GPP management system should have the capability to provide operator configurable filtering capabilities for the telemetry data exchanged between the MnS producer and the MnS consumer.

PREQ-OBS_CNF-Cloud-OAM-08: The 3GPP management system should have the capability to securely expose the filtered observability data to MnS consumers outside of the trusted PLMN domain.

PREQ-OBS_CNF-Cloud-OAM-09: The 3GPP management system should have the capability to securely ingest observability data from MnS producers or consumers outside of the trusted PLMN domain.

PREQ-OBS_CNF-Cloud-OAM-10: The 3GPP management system should have the capability to perform telemetry data correlation between the network functions and data provided by the MnS producers or consumers outside of the trusted PLMN domain.

#### 5.2.7.3	Potential solutions

"OpenTelemetry" [46] could be considered as a potential option for transferring the telemetry data (metrics, events, logs and traces) from the network functions to the unified management system.

#### 5.2.7.4	Evaluation of solutions

The proposed solution in clause 5.2.7.3 describes in high level use of "OpenTelemetry" [46] as a potential option for observability of Network Functions in a cloud native environment. Further study is required in future releases to identify any potential gaps for support of the proposed solution.

## 5.3	Support of different cloud deployment scenarios

### 5.3.1	Use case #1: Placement of cloud native NFs

#### 5.3.1.1	Description

The distributed cloud deployment types enable the network to be deployed across different geographical locations. In this use case CSPs use the information about the available placement locations and resources in each, if available, to select the optimal location(s) for an NF. This provides the NOPs/CSPs with the choice of where a particular NF can be placed at the time of deployment. The parameters that can impact the choice of placement include geographical service area, performance in terms of latency and available bandwidth, as well as level of deployment complexity. In order to support distributed cloud deployments, the 3GPP management system needs to provide the capability for NOPs/CSPs to learn the available deployment locations.

NOTE:	The mechanism to indicate the preferred placement of a particular NF is already supported by use of attribute 'locality' in the ManagedNfProfile datatype.

#### 5.3.1.2	Potential requirements

REQ-1 The 3GPP management system should be able to collect information about the available deployment locations.

NOTE:	The granularity of the location information is FFS.

#### 5.3.1.3	Potential solutions

There are no solutions available for this use case in this document.

#### 5.3.1.4	Evaluation

There are no solution evaluations available for this use case in this document.

# 6	Conclusions and recommendations

## 6.1	General conclusion

The present document mainly studied the following several categories:

-	In clause 4, the terminology and concepts have been studied.

-	In clause 5.1, the use cases are about the configuration, maintenance (i.e., upgrade and traffic management) and policy management for cloud-native VNFs. The related different solutions are based on existing functionalities provided by the 3GPP management system and/or the usage of VNF generic OAM functions/PaaS Services.

-	In clauses 5.2.1, 5.2.3, 5.2.4, 5.2.5 and 5.2.6, the use cases are about the lifecycle management of NF Deployment. The related different solutions can include ETSI NFV MANO, but are not limited to it.

-	In clause 5.2.2, the use cases are about the management data streaming for Network Functions in supporting cloud native.

-	In clause 5.2.7, the use case is about observability for Network Functions in supporting cloud native.

-	In clause 5.3, the use case is about the placement of Network Functions to support different cloud deployment scenarios while there are no solutions available for this use case in this document.

## 6.2	Data streaming for cloud-native NF deployments

It is recommended to further study solution #1 (i.e., data streaming solution based on message bus) presented in clause 5.2.2.3.1 with broader scope in R20 5G-A SBMA topic. The solution#2 presented in clause 5.2.2.3.2 is already supported and no further normative work is required.

## 6.3	Terminology and concepts

It is recommended that future 3GPP normative work specifies the concept and terminology NF Deployment as described in clause 4.2.1 of this document, and clarify the relationship between the term of NF Deployment and VNF.

## 6.4	LCM of NF Deployment

The study has identified use cases, requirements, and solutions in clause 5.2 for 3GPP management system to support LCM of NF Deployment using industry solutions including the use of both ETSI NFV MANO and non ETSI NFV MANO.

To enable 3GPP the use of non ETSI NFV MANO for LCM of NF Deployment in addition to existing use of ETSI NFV MANO solution, it is recommended to start further normative work according to clauses 4.2, 5.2.1, 5.2.3, 5.2.4 and 5.2.5 and enhance TS 28.531 [7] to specify the following in Rel-20:

-	Specify 3GPP management architecture to support LCM of NF Deployment including the requirement for deployment management reference point as described in clause 5.2.1.

-	Specify the use cases and requirements for 3GPP management system to support LCM of NF Deployment as described in clauses 5.2.3, 5.2.4 and 5.2.5.

-	Enhance the LCM of NF procedures in TS 28.531 [7] clauses 7.10, 7.11 and 7.12 to support both NFV-ΜΑΝΟ and Non-ETSI NFV MANO for LCM of NF Deployment, including procedures that decouple the LCM of NF Deployment from LCM of NF.

The following aspects are to be taken into account in the normative work:

-	The 3GPP management system is not responsible for the management of the non-application parameters of the NF Deployment. In the context of the present document, 3GPP management system does not specify non-application parameters of the NF Deployment.

-	Cloud infrastructure management aspects are out of scope of the 3GPP management system.

-	The realization of the deployment management reference point (described in clause 5.2.1.3) is implementation specific and the specification of the interfaces is out of scope of 3GPP.

## 6.5	Use of VNF generic OAM functions

The study has identified use cases that show how VNF generic OAM functions and ETSI NFV MANO can be used to support cloud-native VNFs in terms of non-application level configuration, policy management and upgrade operations. The VNF generic OAM functions and corresponding interfaces specified in ETSI GS NFV-IFA 049 [2]. Their specification is out of scope of 3GPP. The ETSI NFV MANO and corresponding interfaces specified in ETSI GS NFV-IFA 013 [8] and ETSI GS NFV-IFA 008 [9]. Their specification is out of scope of 3GPP.

The application-level configuration, policy management and upgrade operations are performed by the 3GPP management system and the corresponding 3GPP MnSs.

The study also identified a use case and corresponding solution for cloud-native VNF traffic management relying on the traffic enforcer VNF generic OAM function (see clause 5.1.3). However, it is not clear what cloud-native VNF traffic means and the relation with 3GPP signalling traffic. Accordingly, based on the contents in the present document, it is concluded that the traffic management use case is out of scope for 3GPP SA5.

The conclusion is no recommendation could be reached regarding the use of VNF generic OAM functions.

## 6.6	Observability for Network Functions in supporting cloud native

It is recommended to study solutions to address requirements PREQ-OBS_CNF-Cloud-OAM-01 to PREQ-OBS_CNF-Cloud-OAM-10 in 6G.

Annex A:
Example implementation of LCM for NF Deployments using Kubernetes Custom Resource Definitions

This example is one of the possible implementation choices for NF Deployment LCM when using Kubernetes Custom Resource Definition (CRD) defined in "Extend the Kubernetes API with CustomResourceDefinitions |Kubernetes" [47]. Figure A-1 depicts an example scenario for the interaction between the 3GPP management system and the orchestration and management entity for the LCM of a NF Deployment using a Custom Resource (CR) based on Kubernetes® CRD. A CR represents one or more NF Deployments required to realize a part of, or the full functionality of a 3GPP network function.

For this implementation, the NF provisioning MnS producer builds the NF Deployment requirements and uses a Kubernetes client in the producer to create the custom resource (based on Kubernetes® Custom Resource Definition). An authorized Kubernetes client then creates the CR on the Kubernetes cluster. A Kubernetes NF operator is a software extension to Kubernetes that make use of custom resources to manage applications and their components specified in "Operator Pattern | Kubernetes" [48]. Operators follow Kubernetes principles, notably the control loop which helps monitor the CR database for changes. It understands the requirements of the CR and converts them into corresponding native Kubernetes objects.

Figure A-1: Example scenario for interaction with orchestration and management entity for 3GPP NF creation procedure

NOTE:	Figure A-1 depicts interactions within the Kubernetes cluster. These are not meant to be exhaustive and are only depicted for information purposes. Such interactions are neither in scope, nor related to the 3GPP management system.

1.	In Step 1, it is assumed that a NF operator is existing in the Kubernetes cluster. It registers for changes to relevant CRs and waits for updates in its control loop.

2 and 3.	The 3GPP management system builds the NF Deployment(s) requirements and uses a Kubernetes client to invoke interactions with a Kubernetes control plane by using a custom resource based on a Custom Resource Definition.

4.	When a Kubernetes CR object is created in the corresponding control plane, notification updates are sent to the NF operator.

5.	Based on the CR payload, the NF operator creates the relevant Kubernetes objects to realize the NF Deployment(s) in the Kubernetes control plane.

6.	This Kubernetes control plane then creates NF Deployment(s) in the relevant worker nodes.

7.	The Kubernetes control plane updates the status of the NF Deployment(s).

8 and 9.	The NF operator updates the status of the CR to reflect the operational state of the NF Deployment(s).

10 and 11.	The Kubernetes client in the 3GPP management system can query and get the CR status from the Kubernetes cluster in order to check for latest state of the NF Deployment(s).

12.	The NF operator continues to monitor the state of CRs. Upon receiving any further CR updates, the cycle repeats resulting in the modification of the NF Deployment(s).

Annex B:
Example implementations using management data streaming solution based on message bus

As illustrated in figure B-1, the MnS producer implemented in the NF produces management data and sends to the MnS consumer implemented in the MnF via message bus communication. The MnS consumer uses the services exposed by the MnS producer to configure the PM and trace jobs, and convey the end-point information to the MnS producer.

Figure B-1: Example of streaming based on message bus when MnF is embedded in NF

As illustrated in figure B-2, the MnS producer implemented in the MnF A produces management data and sends to the MnS consumer implemented in the MnF B. The MnS consumer uses the services provided by the MnS producer to configure the PM and trace jobs, and convey the end-point information to the MnS producer. The MnS producer reports the management data streaming based on message bus.

Figure B-2: Example of streaming based on message bus when MnF is deployed as a separate entity

Annex C:
Parallel streaming, scaling and resiliency aspects of using management data streaming solution based on message bus

The figure C-1 below illustrates an example implementation using the potential solution for management data streaming based on message bus for scenarios requiring parallel data streaming from NFs with dynamic scaling and resiliency. As shown in figure C-1, the component workload instances of the NFs send data in parallel to the message broker. The number of parallel instances and streaming connections with the message broker can scale in and out dynamically depending on the real-time demand. Furthermore, multiple message broker instances can exist simultaneously with parallel connections to the NFs and MnFs to scale up the transport capacity beyond what a single message broker instance is able to provide. Furthermore, there can be additional redundant message broker instances and connections to provide fault protection and resiliency to the system. The capability for automatic handling of dynamic scaling and resiliency can be provided along with the message broker middleware implementation. This example can be applied to any cloud deployment scenarios, e.g. either at the edge, regional or central cloud.

Figure C-1: Parallel data streaming, scaling and resiliency with solution based on message bus, without data aggregator

Figure C-2 below illustrates another variation of the implementation illustrated in figure C-1 where there is one or more data aggregator(s) which collect and aggregate the data produced by the NFs and their component instances locally before sending it to the message broker(s). The aggregator aggregates the data from multiple data sources (e.g. multiple NFs, or multiple component workload instances of the NFs) first and act as a single point which stream the data to the management system via the message broker(s). This is needed in some deployment scenario when the number of NFs or NF component workload instances are large, and the overhead introduced by the message broking workload for the large number of connections need to be optimized.

Figure C-2: Parallel data streaming, scaling and resiliency with solution based on message bus, with data aggregator

Annex D:
Example scenario using interactions with orchestration and management entity for the creation of NF Deployment instances

Figure D-1 depicts an example scenario for the interaction between the 3GPP management system and the orchestration and management entity for the creation of a NF Deployment instance.

An authorized NF provisioning MnS Consumer requests the creation of a 3GPP NF by creating the ManagedFunction IOC on the NF provisioning MnS producer specifying the network function related requirements. If the 3GPP NF contains containerized or virtualized parts, the NF provisioning MnS producer derives the associated requirements for the NF Deployment based on the network function related requirements.

NOTE 1:	For the network function related requirements, see information model definition for NR NRM in clause 4 and information model definition for 5GC NRM in clause 5 in TS 28.541 [17].

Figure D-1: Example scenario for interaction with orchestration and management entity for 3GPP NF creation procedure

The NF provisioning MnS producer invokes interactions with an orchestration and management entity for the creation of the NF Deployment instance. Following the completion of the required interactions with the orchestration and management entity, the NF provisioning MnS producer creates the MOI for the 3GPP NF instance and configures the new MOI with corresponding information. The NF provisioning MnS producer sends the createMOI response to the NF provisioning MnS Consumer with the DN of the MOI of the 3GPP NF instance.

NOTE 2:	If the orchestration and management entity is ETSI NFV MANO, the interactions over the deployment management reference point are as specified in clause 7.10 of TS 28.531 [7].

Annex E:
Example scenario using interactions with orchestration and management entity for modification of NF Deployment instances

Figure E-1 depicts an example scenario for interaction with orchestration and management entity for modification of an instance of a NF Deployment.

Figure E-1: Example scenario for interaction with orchestration and management entity for modification of an instance of a NF Deployment

An authorized NF provisioning MnS Consumer requests modification of a 3GPP NF instance from the NF provisioning MnS producer by sending a modifyMOIAttributes request with DN of the MOI for the ManagedFunction and the network function related modification requirements. The NF provisioning MnS producer analyses the 3GPP NF instance modification requirements and derives the NF Deployment modification requirements.

The NF provisioning MnS producer invokes interactions with an orchestration and management entity for the modification of the NF Deployment instance. Following the completion of the required interactions with the orchestration and management entity, the NF provisioning MnS producer reconfigures the corresponding MOI for the 3GPP NF instance and sends the modifyMOIAttributes response to the NF provisioning MnS Consumer.

NOTE:	If the orchestration and management entity is ETSI NFV MANO, the interactions over the deployment management reference point are as specified in clause 7.10 of TS 28.531 [7].

Annex F:
Example scenario using interactions with orchestration and management entity for termination of NF Deployment instances

Figure F-1 depicts an example scenario for interaction with an orchestration and management entity for the termination of a 3GPP NF Deployment instance.

Figure F-1: Example scenario for interaction with the orchestration and management entity for the termination of a NF Deployment instance

An authorized NF provisioning MnS Consumer requests the termination of a3GPP NF instance from the NF provisioning MnS producer by sending a deleteMOI request with DN of the 3GPP NF MOI.

The NF provisioning MnS producer responsible for interacting with the orchestration and management (O & M) entity invokes interactions with an O & M entity for the termination of the corresponding NF Deployment instance(s) for the 3GPP NF instance. The NF provisioning MnS producer deletes the MOI of the 3GPP NF instance and sends the deleteMOI response to the NF provisioning MnS Consumer with the DN of the deleted MOI of the 3GPP NF instance. The NF provisioning MnS producer that receives the deleteMOI() request can or cannot be the same NF provisioning MnS producer that interacts with the orchestration and management entity for the termination of the NF Deployment instance.

NOTE:	If the orchestration and management entity is ETSI NFV MANO, the interactions over the deployment management reference point are as specified in clause 7.12 of TS 28.531 [7].

Annex G:
Support for containerized VNF deployments in ETSI NFV

ETSI NFV has developed since Release 4 a set of specifications that enable support for containerized deployments in bare metal and VMs using the NFV-MANO framework. The various dimensions considered include aspects such as containerized VNF lifecycle management, descriptors' management, containerized VNF onboarding and containerized cluster management.

NOTE:	In ETSI NFV specifications, the term VNF also covers the case of a cloud native NF, as the VNF modelling is also able to cater for the specific design cloud-native and containerization properties. Furthermore, the terms virtualization and cloudification are used interchangeably, while the terms containerized NF, container based VNF and containerized VNF refer to the same concept and are also used interchangeably.

The NFV-MANO architectural framework introduced in Release 4 supports containerized VNF deployments and is compatible with the de facto Kubernetes® open-source solutions. Additionally, it remains compatible with previous ETSI NFV Releases that offer services for VM-based deployments. The NFV-MANO architectural framework with support for containers is specified in ETSI GS NFV 006 [18].

Table G-1 summarizes the key ETSI NFV specifications for supporting containerized VNF deployments.

Table G-1: Summary of NFV-MANO specifications enabling support for containers.

| Stage | Specification | Topic |
| --- | --- | --- |
| Stage 0 (info) | ETSI GR NFV-IFA 029 [25] | Use cases, concepts, and recommendations. |
| Stage 1 & 2 | ETSI GS NFV-IFA 040 [28] (new in Rel-4) ETSI GS NFV-IFA 036 [26] (new in Rel-4) | Containerized workloads management and orchestration, container cluster management and orchestration (requirements, functionality, object modelling, etc.) |
|  | ETSI GS NFV-IFA 010 [21] | Enhanced NFV-MANO functional requirements including functional requirements for existing NFV-MANO functional blocks and new NFV-MANO functions responsible for the management and orchestration of containerized workloads and container clusters. |
|  | ETSI GS NFV-IFA 007 [19] ETSI GS NFV-IFA 008 [20] ETSI GS NFV-IFA 013 [23] | Enhanced interfaces, and information models for functionality produced by existing NFV-MANO functional blocks, such as NFVO and VNFM. |
|  | ETSI GS NFV-IFA 011 [22] ETSI GS NFV-IFA 014 [24] | Information modelling of enhanced descriptors/templates. |
| Stage 3 | ETSI GS NFV-SOL 018 [36] (new in Rel-4) | Mapping of the NFV object model for OS container management to Kubernetes® managed objects. Protocol and data models profiling Kubernetes® APIs fulfilling the requirements specified in respective stage 2 documents (ETSI GS NFV-IFA 040 [28] and ETSI GS NFV-IFA 036 [26]). |
|  | ETSI GS NFV-SOL 020 [37] (new in Rel-4) | Protocol and data models profiling Cluster API (CAPI) fulfilling the requirements specified in respective stage 2 document (ETSI GS NFV-IFA 036 [26]) regarding cluster management. |
|  | ETSI GS NFV-SOL 001 [30] ETSI GS NFV-SOL 004 [33] | Data model (stage 3) of enhanced descriptors and packaging supporting containerized workloads. |
|  | ETSI GS NFV-SOL 002 [31] ETSI GS NFV-SOL 003 [32] ETSI GS NFV-SOL 005 [34] | Protocol and data models (stage 3) of enhanced NFV-MANO APIs. |

Additional aspects, such as networking for container-based deployments and creation of secondary cluster networks are investigated in ETSI GR NFV-IFA 038 [27] and ETSI GR NFV-IFA 043 [29]. Performance measurements of containerized deployments are specified in ETSI GS NFV-IFA 027 [38]. Study of power efficiency topics are addressed in ETSI GR NFV-EVE 021 [39]. Topics related to physical infrastructure management for both VM and containerized deployments (in bare metal or VMs) are covered in ETSI GS NFV-IFA 053 [40].

Table G-2 summarizes the key ETSI NFV terms used when considering operations in containerized deployments and the corresponding terms used in Kubernetes®.

Table G-2: Terminology alignment for containerized VNFs

| ETSI NFV term | Kubernetes ecosystem term |
| --- | --- |
| Container Infrastructure Service (CIS) | Kubernetes® services exposing CRI, CNI, CSI |
| Container Infrastructure Service (CIS) instance | Kubernetes® node |
| Container Infrastructure Service Management (CISM) | Kubernetes® control plane & Helm 3 client |
| CIS cluster | Kubernetes® cluster |
| Managed Container Infrastructure Object (MCIO) | Kubernetes® managed objects (e.g. Deployments, StatefulSet, Persistent Volume Claim, Service, etc.) |
| Managed Container Infrastructure Object Package (MCIOP) | Helm charts |
| Managed CIS Cluster Object (MCCO) | Kubernetes® Custom Resource Definition (CRD), etc. |
| Container Image Repository (CIR) | Docker Registry |
| CIS Cluster Management (CCM) | Cluster API (CAPI) |

Annex H:
Support for CNF deployments in ETSI NFV

# H.1	Create CNF example using NFV-MANO

In TS 23.531 [x] the flow and procedures of creating network function instances are described, through instantiation of the VNF(s) that realize the virtualized part of subject 3GPP NF, using NFV-MANO.

In clause 5.2.3 of the present document, use case #3 is about the creation of an NF Deployment instance running in the cloud.

In this Annex details are provided regarding the operations performed in NFV-MANO upon an instantiation request for the case of NF Deployments in containers using NFV-MANO. The process of creation of cloud native VNFs using NFV-MANO procedures is described in ETSI GS NFV-SOL 016 [35] and is summarized as follows.

Assuming there is no MCIOP available in the NFV-MANO system, the 3GPP management system interacts with NFVO over the Os-Ma-nfvo reference point using the VNF Package Management interface described in ETSI GS NFV-SOL 005 [34] to provide the VNF Package. The VNF Package is described in ETSI GS NFV-SOL 004 [33] and contains the VNFD and additional files (like a manifest file, files used for testing, non-MANO artifacts like scripts etc.). The VNFD of a cloud native VNF has references to one or multiple MCIOPs, which are included as file artifacts in the VNF Package (see ETSI GS NFV-SOL 001 [30]).

NOTE 1:	In ETSI NFV MCIO stands for Managed Container Infrastructure Object and MCIOP stands for Managed Container Infrastructure Object Package. See clause 4 for more details on support for CNF deployments in ETSI NFV.

As an example, in case the cloud native VNF to be deployed refers to a Kubernetes Deployment (or a Kubernetes Pod etc.), then the corresponding entity in NFV-MANO is a Compute MCIO that needs to be instantiated. The MCIO declarative descriptor (i.e., the Kubernetes® manifest file) needed is in the MCIOP. The availability of the MCIOP in the local MCIOP repository does not mean instantiation of the MCIO.

NOTE 2:	The process of uploading the VNF Package described above can be also triggered when uploading a corresponding NSD archive file.

To trigger the instantiation of the cloud native VNF as part of an NS, the 3GPP management system needs to request the instantiation of a corresponding NS by calling the Instantiate NS operation exposed by NFVO over Os-Ma-nfvo. For the instantiation of the cloud native NF, VNFM fetches from the NFVO a VNFD (by passing the vnfPkgId) (the VNFD is available in the system according to the process described before). Then by analysing the VNFD and performing the VNF LCM granting procedure between the VNFM and the NFVO, the VNFM requests to CISM through the OS container workload management service interface, to instantiate and configure the containerized workload part of the VNF as described in clause 7.3.1 of ETSI GS NFV-SOL 018 [36]. The CISM creates the MCIO (i.e., the Kubernetes® Deployment) by interpreting the MCIOP available to the local MCIOP repository and allocating the requested infrastructure resources on CIS instances. Other aspects like configuration of a secondary container cluster network are achieved through interaction of CISM and CCM with the NFVO.

It is also possible that the 3GPP management system requests the instantiation directly to the VNFM by interacting on the VNFM exposed interfaces over the Ve-Vnfm-em reference point, considering that the NFV-MANO framework also enables NS-level procedures to be decoupled from VNF-level ones. For instance, an NS managed object can be prepared in advanced by interacting with the NFVO but leaving such an NS "empty"; afterwards, VNF instantiation can be triggered directly to the VNFM, and later add the VNF instance into the NS by interacting with the NFVO.

Annex I (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-01 | SA5#153 | S5-241053 | - |  | - | Initial skeleton | 0.0.0 |
| 2024-02 | SA5#153 | S5-241054 |  |  |  | Add TR structure | 0.1.0 |
| 2024-04 | SA5#154 | S5-241961 |  |  |  | Add Scope | 0.2.0 |
|  |  | S5-241962 |  |  |  | Add terms for VNF generic OAM function |  |
|  |  | S5-241963 |  |  |  | Add Background of VNF generic OAM functions |  |
|  |  | S5-241874 |  |  |  | Add new use case description on Cloud-native VNF configuration management |  |
|  |  | S5-241970 |  |  |  | Add new use case 3GPP management architecture to support LCM of cloud native network function |  |
|  |  | S5-242164 |  |  |  | Add new use case description on data streaming for cloud native NFs |  |
| 2024-06 | SA5#155 | S5-243182 |  |  |  | pCR 28.869 Terminology alignment | 0.3.0 |
|  |  | S5-243185 |  |  |  | pCR 28.869 Add Summary of TR 28.834 use cases related to generic OAM functions |  |
|  |  | S5-243184 |  |  |  | pCR 28.869 VNF generic OAM functions and SBMA analysis |  |
|  |  | S5-243187 |  |  |  | pCR 28.869 Add use cases of Cloud-native VNF policy management |  |
|  |  | S5-243424 |  |  |  | pCR 28.869 Configuration manager requirements |  |
|  |  | S5-243191 |  |  |  | pCR TR 28.869 Enhance use case description and add potential requirements for data streaming for cloud-native network functions |  |
|  |  | S5-243192 |  |  |  | pCR TR 28.869 Add a potential solution for data streaming for cloud-native network functions |  |
|  |  | S5-243190 |  |  |  | Adding solutions and requirements for data streaming for cloud native NFs |  |
|  |  | S5-243189 |  |  |  | pCR 28.869 New use case on creation of cloud native NFs |  |
|  |  | S5-243195 |  |  |  | pCR 28.869 New use case on modification of cloud-native NFs |  |
|  |  | S5-243196 |  |  |  | pCR 28.869 New use case on termination of cloud-native NFs |  |
|  |  | S5-243198 |  |  |  | pCR 28.869 Concepts and background relevant to WT-3 |  |
|  |  | S5-243194 |  |  |  | pCR 28.869 New use case on placement of cloud-native NFs |  |
|  |  | S5-243201 |  |  |  | pCR 28.869 Support for containers in NFV |  |
|  |  | S5-243197 |  |  |  | pCR 28.869 Create CNF example using MANO |  |
| 2024-08 | SA5#156 | S5-244698 |  |  |  | Rel-19 pCR TR 28.869 Add background info on cloud-native design principles and their relevance to 3GPP OAM | 0.4.0 |
|  |  | S5-244701 |  |  |  | pCR Add Background of cloud-native VNFs |  |
|  |  | S5-244700 |  |  |  | pCR TR 28.869 concept and terminology |  |
|  |  | S5-244702 |  |  |  | pCR 28.869 VNF Config Manager solutions and evaluation |  |
|  |  | S5-244841 |  |  |  | pCR Add potential solutions for Cloud-native VNF policy management |  |
|  |  | S5-244851 |  |  |  | pCR 28.869 cloud-native VNF Traffic management UC and requirements |  |
|  |  | S5-244852 |  |  |  | pCR Add potential solutions for Cloud-native VNF traffic management |  |
|  |  | S5-244849 |  |  |  | pCR 28.869 Upgrade cloud-native VNF UC, requirements and solution |  |
|  |  | S5-244847 |  |  |  | pCR TR 28.869 remove editor note and adopt to new concept in use case1 |  |
|  |  | S5-244848 |  |  |  | pCR TR 28.869 remove editor note and adopt to new concept in use case3 |  |
|  |  | S5-244842 |  |  |  | pCR TR 28.869 Add use case and solutions for LCM of cloud native NF based on standard K8S CRD models |  |
|  |  | S5-244844 |  |  |  | pCR TR 28.869 Enhance potential requirement for message bus based data streaming use case to support tracing and analytic data |  |
|  |  | S5-244843 |  |  |  | pCR TR 28.869 Adding new implementation example in Annex for message bus-based data streaming with data aggregator |  |
|  |  | S5-244845 |  |  |  | Rel-19 pCR TR 28.869 Add use case for scaling cloud-native NFs |  |
|  |  | S5-244846 |  |  |  | Rel-19 pCR TR 28.869 Add potential solution and solution evaluation for scaling cloud-native NFs use case |  |
| 2024-09 | SA#105 | SP-241129 |  |  |  | Presented at SA#105 for Information | 1.0.0 |
| 2024-09 |  |  |  |  |  | After editHelp cleanup | 1.0.1 |
| 2024-10 | SA5#157 | S5-245812 |  |  |  | pCR TR 28.869 Removing unnecessary statement regarding the consumers of the VNF generic OAM functions | 1.1.0 |
|  |  | S5-246005 |  |  |  | pCR 28.869 terminology related ENs |  |
|  |  | S5-246117 |  |  |  | pCR 28.869 VNF Configuration management ENs |  |
|  |  | S5-246121 |  |  |  | pCR 28.869 Add details for termination of a NF deployment instance |  |
|  |  | S5-246123 |  |  |  | Rel-19 pCR TR 28.869 Resolve editor's note in Annex D by updating the content and the related figure |  |
|  |  | S5-246124 |  |  |  | Rel-19 pCR TR 28.869 Resolve editor's note in Annex E by updating the content and the related figure |  |
|  |  | S5-246240 |  |  |  | pCR TR 28.869 Update Use cases for use of VNF generic OAM functions |  |
|  |  | S5-246241 |  |  |  | pCR TR 28.869 Add potential deployment scenario for using VNF generic OAM functions in SBMA |  |
|  |  | S5-246242 |  |  |  | pCR 28.869 addressing Annexes_ENs |  |
|  |  | S5-246243 |  |  |  | pCR 28.869 Add evaluation of solutions for creation of a NF eployment instance |  |
|  |  | S5-246244 |  |  |  | pCR 28.869 note for data streaming services |  |
|  |  | S5-246297 |  |  |  | pCR TR 28.869 remove editor note and adopt to new concept in use case 5.2.4 |  |
| 2024-11 | SA5#158 | S5-247104 |  |  |  | pCR TR 28.869 Add terminology for descriptor | 1.2.0 |
|  |  | S5-247107 |  |  |  | pCR TR28.869 Cloud-native VNF Configuration management evaluation |  |
|  |  | S5-247110 |  |  |  | pCR Fix normative text in TR |  |
|  |  | S5-247113 |  |  |  | Rel-19 pCR TR 28.869 Resolve editor's note in Annex F and add note to Annex D and Annex E |  |
|  |  | S5-247124 |  |  |  | pCR TR 28.869 Modify requirements, add new potential solution evaluation 4 data streaming for cloud native NF |  |
| 2025-02 | SA5#159 | S5-250972 |  |  |  | pCR TR 28.869 enhance use case description for data streaming for cloud native network function | 1.3.0 |
|  |  | S5-250950 |  |  |  | Rel-19 pCR 28.869 Add description of specification impacts |  |
|  |  | S5-250868 |  |  |  | pCR TR28.869 VNF configuration management |  |
|  |  | S5-250866 |  |  |  | R19 pCR TR 28.869 add Observability for CNFs |  |
|  |  | S5-250865 |  |  |  | pCR TR 28.869 Add clarification to terminology |  |
| 2025-04 | SA5#160 | S5-252012 |  |  |  | pCR TR 28.869 Adopt NF Deployment terminology in scaling use case | 1.4.0 |
|  |  | S5-251891 |  |  |  | pCR TR 28.869 enhance observability requirements (Use case 7) |  |
|  |  | S5-252010 |  |  |  | Pseudo-CR on TR 28.869 Add an example in Annex B |  |
|  |  | S5-251579 |  |  |  | pCR Add solution and evaluation statement for usecase Placement of cloud native NFs |  |
|  |  | S5-252014 |  |  |  | pCR Add general conclusion and some recommendations |  |
|  |  | S5-251892 |  |  |  | pCR Add evaluation for solutions in clause 5.2.1.3 |  |
|  |  | S5-251893 |  |  |  | pCR TR 28.869 Annex example for CRD based deployment |  |
| 2025-05 | SA5#161 | S5-252622 |  |  |  | Pseudo-CR on TR28.869 VNF generic OAM functions and PaaS Services relationship | 1.5.0 |
|  |  | S5-252858 |  |  |  | pCR TR 28.869 Add potential solution for VNF configuration |  |
|  |  | S5-252991 |  |  |  | pCR on TR 28.869 Add conclusion for management data streaming solution based on message bus |  |
|  |  | S5-252619 |  |  |  | Pseudo-CR on TR28.869 editorial cleanup removing ENs |  |
|  |  | S5-252513 |  |  |  | Pseudo-CR on TR 28.869 clean up about editorial and small corrections |  |
| 2025-07 |  |  |  |  |  | MCC: Previous Word file was corrupted. | 1.5.1 |
| 2025-08 | SA5#162 | S5-254015 |  |  |  | Rel-19 pCR TR 28.869 Editorial clean up-Clause 5.2.3.3.2, Annexes D, E and F | 1.6.0 |
|  |  | S5-254016 |  |  |  | pCR TR 28.869 Updates to declarative descriptor-based LCM |  |
|  |  | S5-254017 |  |  |  | Pseudo-CR on TR 28.869 Add evaluation for some LCM solutions |  |
|  |  | S5-254020 |  |  |  | Pseudo-CR-TR 28.869 Add recommendations for terminology |  |
|  |  | S5-254021 |  |  |  | pCR TR 28.869 Add evaluation and conclusion for management data streaming based on message bus |  |
| 2025-10 | SA5#163 | S5-254734 |  |  |  | Pseudo-CR on TR28.869 Config management updates | 1.7.0 |
|  |  | S5-254879 |  |  |  | Pseudo-CR on TR28.869 Terminology alignment |  |
|  |  | S5-254635 |  |  |  | Rel-19 pCR TR 28.869 Add Evaluation to the use of VNF generic OAM functions |  |
|  |  | S5-254636 |  |  |  | Rel-19 pCR TR 28.869 Add conclusions and recommendations to the use of VNF generic OAM functions |  |
|  |  | S5-254637 |  |  |  | Pseudo-CR-TR 28.869 Add evaluation for Observability solutions |  |
|  |  | S5-254735 |  |  |  | Pseudo-CR-TR 28.869 Add Rapporteur clean-up and solve some editor's notes |  |
|  |  | S5-254633 |  |  |  | pCR TR 28.869 Recommendation for LCM of NF Deployment |  |
| 2025-12 | SA#110 | SP-251413 |  |  |  | Presentation to TSG SA for approval | 2.0.0 |