3GPP TSG-CT Meeting #109	CP-252192

Beijing, P.R. China; 15th – 16th September 2025	was C4-253459

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.572 | CR | 0367 | rev | 2 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve Editor's Notes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson, vivo |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-09-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-19, two Editor's Notes were added for AIML Positioning in TS 29.572: Editor's Note: Whether more cause codes to be defined is FFS. Analysis: Besides the "not enough sampling" cause code, stage 2 didn't define new cause codes for AIML Positioning in Rel-19. The editor's note can be reverted. New cause code(s) was determined/defined in future if needed. Editor's note: The data samples used for LMF-based AI/ML Positioning will be decided by RAN WGs. Analysis: In Rel-19, only RAN is supported as data-source (UE is excluded). The sampling data from RAN was defined by reuing the location measurement data type from LMF Location API, which is with Bytes type (base64 encoded string). There will be no binary data for LMF Data Exposure API in Rel-19. The binary data type clause can be reverted. This CR also fixes a typo in table title in 6.1.6.2.62. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1/ Resolve the Editor's Note by removal. 2/ Revert the binary data type clause for LMF Data Exposure API. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Invalid Editor's Note in specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.62, 6.3.6.3.3, 6.3.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact any yaml file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: - Keep clause 6.3.6.4 with statement that no binary data types are defined in this release. - Add Vivo as co-source. Rev2: Add missed second change |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.1.6.2.62	Type: AddLocationContext

Table 6.1.6.2.62-1: Definition of type AddLocationContext

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| locationQoS | LocationQoS | C | 0..1 | This IE shall contain the location QoS if available. |
| ldrType | LdrType | M | 1 | The type of LDR |
| hgmlcCallBackURI | Uri | M | 1 | Callback URI of the H-GMLC |
| ldrReference | LdrReference | M | 1 | LDR Reference |
| periodicEventInfo | PeriodicEventInfo | C | 0..1 | Information for periodic event reporting. (NOTE 1) |
| areaEventInfo | AreaEventInfo | C | 0..1 | Information for area event reporting. (NOTE 1) |
| motionEventInfo | MotionEventInfo | C | 0..1 | Information for motion event reporting. (NOTE 1) |
| eventReportMessage | EventReportMessage | C | 1 | This IE shall be present when the event report message from the UE is available for the location context being transferred to the target LMF. When present, this IE shall contain the event report message received from the UE for the location context. |
| eventReportingStatus | EventReportingStatus | O | 0..1 | Status of event reporting |
| ueLocationInfo | UELocationInfo | O | 0..1 | Location information for the target UE |
| scheduledLocTime | DateTime | O | 0..1 | When present, this IE shall contain the scheduled time (in UTC) that the UE needs to be located. |
| indoorOutdoorInd | IndoorOutdoorInd | O | 0..1 | When present, this IE shall indicate whether the UE is indoor or outdoor. |
| losNlosMeasureInd | LosNlosMeasureInd | O | 0..1 | When present, this IE shall indicate whether LOS measurement or NLOS measurement is used. |
| upCumEvtRptCriteria | UpCumEvtRptCriteria | O | 0..1 | When present, this IE shall include the cumulative event report timer or the maximum number of location reporting over user plane or both. |
| lcsUppExistInd | boolean | C | 0..1 | This IE shall be present with the value true, if the location context being transferred is associated with the LCS-UPP connection between the UE and the source LMF. Presence of this IE with the value false shall be prohibited. |
| NOTE 1: At least one of periodicEventInfo, areaEventInfo or motionEventInfo shall be present in the LocContextList structure. |  |  |  |  |

* * * Next Change * * * *

##### 6.3.6.3.3	Enumeration: LmfDataExposureCause

The enumeration LmfDataExposureCause represents the cause code for the LMF Data Exposure reports.

Table 6.3.6.3.3-1: Enumeration LmfDataExposureCause

| Enumeration value | Description |
| --- | --- |
| "NOT_ENOUGH_SAMPLES" | Indicates that the requested number of sampling data cannot be met within the Area of Interest during the requested time window. |

* * * Next Change * * * *

#### 6.3.6.4	Binary data types

In this release of this specification, no binary data types are defined.

* * * End of Changes * * * *