3GPP TSG CT WG3 Meeting #143	C3-254322

Sophia-Antipolis, France, 13 – 17 October 2025	(Revision of C3-25xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.575 | CR | 0119 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Supported Features |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc19 |  |  |  |  |  | Date: |  |  | 2025-09-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The table description on the presence condition for the "suppFeat" attribute are not correct in the affected clauses, need to be corrected as the descriptions agreed in CT3#142. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected table descriptions for the "suppFeat" attribute with correct condition description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not correct descriptions for supported features in the impacted clause will arouse wrong implementation issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.6.2.2, 5.1.6.2.3, 5.1.6.2.4, 5.1.6.2.6, 5.2.6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact the OpenAPI file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

##### 5.1.6.2.2	Type: NadrfDataStoreRecord

Table 5.1.6.2.2-1: Definition of type NadrfDataStoreRecord

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataNotif | DataNotification | C | 0..1 | Data subscription notification. (NOTE 1) |  |
| anaNotifications | array(NnwdafEventsSubscriptionNotification) | C | 1..N | List of analytics subscription notifications. (NOTE 1) |  |
| anaSub | NnwdafEventsSubscription | C | 0..1 | Represents the subscription information of the corresponding analytics notification. Shall be present if the "anaNotifications" attribute is provided. (NOTE 2) |  |
| dataSetTag | DataSetTag | O | 0..1 | Data set tag of the stored data or analytics. | EnhDataMgmt |
| dsc | string | O | 0..1 | Data synthesis and compression information. (NOTE 3) | EnhDataMgmt |
| dataSub | DataSubscription | C | 0..1 | Represents the subscription information of the corresponding data notification. Shall be present if the "dataNotif" attribute is provided. (NOTE 2) |  |
| storeHandl | StorageHandlingInfo | O | 0..1 | Contains storage handling information for the data or analytics that are being stored. | EnhDataMgmt |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 5.1.8. This attribute shall be provided only when feature negotiation needs to take place. |  |
| NOTE 1: Exactly one of the attributes "anaNotifications" and "dataNotif" shall be provided. NOTE 2: Exactly one of the attributes "anaSub" and "dataSub" shall be provided. NOTE 3: The format and the contents of this attribute are up to the implementation. |  |  |  |  |  |

# *** 2nd Change ***

##### 5.1.6.2.3	Type: NadrfDataStoreSubscription

Table 5.1.6.2.3-1: Definition of type NadrfDataStoreSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| anaSub | NnwdafEventsSubscription | C | 0..1 | Subscribed analytics events. (NOTE 1) |  |
| dataSetTag | DataSetTag | O | 0..1 | Data set tag of the stored data or analytics. | EnhDataMgmt |
| dataSub | DataSubscription | C | 0..1 | Represents requested Events subscription. (NOTE 1) |  |
| targetNfId | NfInstanceId | C | 0..1 | DCCF or NWDAF NF instance identifier to which the ADRF shall create the requested subscription. (NOTE 2) |  |
| targetNfSetId | NfSetId | C | 0..1 | DCCF or NWDAF NF set identifier to which the ADRF shall create the requested subscription. (NOTE 2) |  |
| formatInstruct | FormattingInstruction | O | 0..1 | Formatting instructions to be used for sending event notifications. |  |
| procInstruct | ProcessingInstruction | O | 0..1 | Processing instructions to be used for sending event notifications. (NOTE 3) |  |
| multiProcInstructs | array(ProcessingInstruction) | O | 1..N | Processing instructions to be used for sending event notifications. (NOTE 3) | MultiProcessingInstruction |
| storeHandl | StorageHandlingInfo | O | 0..1 | Contains storage handling information for the data or analytics that will be collected and stored based on the requested subscription. | EnhDataMgmt |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 5.1.8. This attribute shall be provided only when feature negotiation needs to take place. |  |
| NOTE 1: Exactly one of these attributes shall be provided. NOTE 2: One of "targetNfId" and "targetNfSetId" shall be provided. NOTE 3: The "multiProcInstructs" attribute shall be used instead of the "procInstruct" attribute when the "MultiProcessingInstruction" feature is supported. |  |  |  |  |  |

# *** 3rd Change ***

##### 5.1.6.2.4	Type: NadrfDataRetrievalSubscription

Table 5.1.6.2.4-1: Definition of type NadrfDataRetrievalSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| anaSub | NnwdafEventsSubscription | C | 0..1 | Subscribed analytics events. (NOTE 1) |  |
| dataSetId | string | C | 0..1 | Data set identifier of stored data or analytics records. (NOTE 1) | EnhDataMgmt |
| dataSub | DataSubscription | C | 0..1 | Represents requested Events subscription. (NOTE 1) |  |
| notificationURI | Uri | M | 1 | Notification target address. |  |
| timePeriod | TimeWindow | M | 1 | Represents a start time and a stop time during which the requested data is collected and/or will be collected at the data source. |  |
| notifCorrId | string | M | 1 | Notification correlation identifier provided by the NF service consumer to be used later by the ADRF in the notifications that correspond with this subscription. The value of this attribute shall be unique per subscription for a given NF service consumer. |  |
| consTrigNotif | boolean | O | 0..1 | If provided and set to "true", it indicates that notifications shall be buffered (sending only fetch instructions to the NF service consumer) until the NF service consumer requests their delivery using Nadrf_DataManagement Service. The default value is "false". |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 5.1.8. This attribute shall be provided only when feature negotiation needs to take place. |  |
| NOTE 1: Exactly one of these attributes shall be provided. |  |  |  |  |  |

# *** 4th Change ***

##### 5.1.6.2.6	Type: NadrfDataStoreSubscriptionRef

Table 5.1.6.2.6-1: Definition of type NadrfDataStoreSubscriptionRef

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| transRefId | string | C | 0..1 | Transaction reference identifier. (NOTE) |  |
| dataSetId | string | C | 0..1 | Data set identifier associated with a storage subscription. (NOTE) | EnhDataMgmt |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 5.1.8. This attribute shall be provided only when feature negotiation needs to take place. |  |
| NOTE: One of the "transRefId" and "dataSetId" attributes shall be provided. |  |  |  |  |  |

# *** 5th Change ***

##### 5.2.6.2.2	Type: NadrfMLModelStoreRecord

Table 5.2.6.2.2-1: Definition of type NadrfMLModelStoreRecord

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| nfInstanceId | NfInstanceId | C | 0..1 | NF instance Identifier of the NWDAF containing MTLF. (NOTE 1) |  |
| nfSetId | NfSetId | C | 0..1 | NF set identifier of the NWDAF containing MTLF. (NOTE 1) |  |
| mlModelInfo | array(MLModelInfo) | C | 1..N | ML Model information. (NOTE 2) |  |
| mlModels | array(MLModel) | C | 1..N | Each element represents an ML model. (NOTE 2) |  |
| modelStoreResult | ModelStoreResult | O | 0..1 | Indicates the result of the store operation. If the "EnModelMgmt" feature is supported, the attribute also indicates the failure cause of the retrieval operation. (NOTE 3) |  |
| modelStoreResults | array(ModelStoreResult) | O | 1..N | Indicates the result of the store operation. (NOTE 3) | EnModelMgmt |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 5.2.8. This attribute shall be provided only when feature negotiation needs to take place. |  |
| NOTE 1: One of "nfInstanceId" and "nfSetId" attributes shall be provided. NOTE 2: Any of the "mlModelInfo" and "mlModels" attributes shall be provided. NOTE 3: The "modelStoreResult" and "modelStoreResults" attributes are only applicable to the response to an Nadrf_MLModelManagement_StorageRequest service operation. The "modelStoreResults" attribute shall be used instead of the "modelStoreResult" attribute when the "EnModelMgmt" feature is supported. If the "EnModelMgmt" feature is supported, this attribute is also applicable to the response to an Nadrf_MLModelManagement_RetrievalRequest service operation. |  |  |  |  |  |

*** End of Changes ***