3GPP TSG-CT WG3 Meeting #143	C3-254395

# Sophia-Antipolis, FR, 13th – 17th October, 2025	                                   (Revision of C3-254209)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.507 | CR | 0356 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding the OAM information as one input of policy decision |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, eNS_Ph3, IIoT |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As indicated in the text below from 4.2.4.2, the policy decisions made by the PCF may be based on the information from the OAM. - when the PCF detects that one or more S-NSSAI(s) of the UE's Allowed NSSAI and/or Partially Allowed NSSAI become(s) unavailable for a UE based on an OAM trigger, a received NWDAF notification or PCF internal triggers, the PCF may indicate this to the AMF by providing the "snssaiReplInfos" attribute containing these impacted S-NSSAI(s), and for each impacted S-NSSAI, the corresponding status information set to "UNAVAILABLE" and optionally an Alternative S-NSSAI; and |  |  |  |  |  |  |  |  |
| Summary of change: |  | 4.1.3.1 update to add the information from OAM and SM PCF as two additional input of policy decision. Add abbreviations for OAM and TSCTSF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 4.1.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not have any impact in the OpenAPI specification. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5G-BRG	5G Broadband Residential Gateway

5G-RG	5G Residential Gateway

5GC	5G Core Network

5G-CRG	5G Cable Residential Gateway

5GS	5G System

AMBR	Aggregated Maximum Bit Rate

AMF	Access and Mobility Management Function

BBF	Broadband Forum

CHF	Charging Function

DNN	Data Network Name

EPC	Evolved Packet Core

EPS	Evolved Packet System

E-UTRAN	Evolved Universal Terrestrial Radio-Access Network

FN-BRG	Fixed Network Broadband Residential Gateway

FN-CRG	Fixed Network Cable Residential Gateway

FN-RG	Fixed Network Residential Gateway

FQDN	Fully Qualified Domain Name

GBR	Guaranteed Bit Rate

GPSI	Generic Public Subscription Identifier

GUAMI	Globally Unique AMF Identifier

HFC	Hybrid Fiber-Coaxial

JSON	JavaScript Object Notation

LBO	Local Break Out (roaming)

MBR	Maximum Bit Rate

MME	Mobility Management Entity

NID	Network Identifier

NRF	Network Repository Function

NSSAI	Network Slice Selection Assistance Information

NWDAF	Network Data Analytics Function

OAM	Operation, Administration and Maintenance

PCF	Policy Control Function

PEI	Permanent Equipment Identifier

PRA	Presence Reporting Area

QoS	Quality of Service

RA	Registration Area

RFSP	RAT Frequency Selection Priority

SMF	Session Management Function

S-NSSAI	Single Network Slice Selection Assistance Information

SNPN	Stand-alone Non-Public Network

SUPI	Subscription Permanent Identifier

TSCTSF	Time Sensitive Communication and Time Synchronization Function

UDM	Unified Data Management

URSP	UE Route Selection Policy

V-PCF	Visited Policy Control Function

W-5GAN	Wireline 5G Access Network

W-5GBAN	Wireline BBF Access Network

W-5GCAN	Wireline 5G Cable Access Network

W-AGF	Wireline Access Gateway Function

# *** 2nd Change ***

4.1.3.1	Policy Control Function (PCF)

The Policy Control Function (PCF):

-	Supports unified policy framework to govern network behaviour; and

-	Provides Access and Mobility Management related policies to the NF service consumer that enforces them.

In the roaming scenario, the Visited Policy Control Function (V-PCF) provides the functions described in this clause towards the visited network.

The policy decisions made by the PCF may be based on one or more of the following:

-	Information obtained from the AF/NEF, e.g. high throughput indication;

-	Information obtained from the UDR;

-	Information obtained from the AMF, e.g. UE related and access related information;

-	Information obtained from the NWDAF;

-	Information from the CHF about spending limit control;

-	Information from the OAM System;

-	Information from the PCF for a PDU session;

-	Information from the TSCTSF; and

-	PCF pre-configured policy context.

*** End of Changes ***