# 3GPP TSG CT WG3 Meeting #143	C3-254545

# Sophia-Antipolis, France, 13 – 17 October, 2025		 (Revision of C3-254252, 4497)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.521 | CR | 0228 | rev | 2 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PCF for UE binding clean up |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, TEI17_DCAMP |  |  |  |  |  | Date: |  |  | 2025-9-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In 5G, NRF provides an NFStatus service for the consumer to supervise the status change of an NF or NF service and TS 23.527 describes the information flow for notifying the consumer about subscribed NF failure/restart cases. Alternatively, TS 23.527 subclause 6.4.2 allows the direct signalling between consumer and producer to detect a specific consumer restart. The BSF binding resource is created upon successful PCF request, and if PCF has experienced failure /restart, usually a recovery time is associated with the PCF. However, such recovery time is missing for the current PCF for UE binding. In current specification, the PCF for PDU session and PCF for MBS session, the recovery time is provided and it also has meaning to provide such information for the PCF for UE binding. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add PCF recovery time in the PcfForUeBinding data type. It is up to BSF to decide which method(s) to use for resource cleanup. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The BSF is unable to detect PCF restart so to clean up the resource. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.3, 5.6.2.10, 5.8, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible corrections to the following APIs: TS29521_Nbsf_Management.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

#### 4.2.2.3	Register a new PCF for a UE binding information

Figure 4.2.2.3-1: NF service consumer registers a new PCF for a UE binding information

The NF service consumer shall invoke the Nbsf_Management_Register service operation to register the PCF for a UE binding information in the BSF. The NF service consumer shall send for this an HTTP POST request with "{apiRoot}/nbsf-management/<apiVersion>/pcf-ue-bindings" as Resource URI representing the "PCF for a UE Bindings", as shown in figure 4.2.2.3-1, step 1, to create a binding information for an "Individual PCF for a UE Binding" according to the information in the message body.

The "PcfForUeBinding" data structure included in the request message body shall include:

-	SUPI encoded as "supi" attribute; and

-	if the PCF supports the Npcf_AMPolicyAuthorization service, the Npcf_AMPolicyAuthorization service address information consisting of:

a.	the FQDN of the PCF encoded as "pcfForUeFqdn" attribute; and/or

b.	a description of IP endpoints at the PCF hosting the Npcf_AMPolicyAuthorization service encoded as "pcfForUeIpEndPoints" attribute;

NOTE:	In this release of the specification the PCF for a UE registering the binding information in the BSF supports the Npcf_AMPolicyAuthorization service.

and may include:

-	GPSI encoded as "gpsi" attribute;

-	PCF instance Id in "pcfId" attribute;

-	if the "Recovery" feature is supported, the recovery timestamp of the NF service consumer, within the "recoveryTime" attribute;

-	the PCF Set identifier in the "pcfSetId" attribute; and

-	the binding level in the "bindLevel" attribute.

If the NF service consumer (e.g., PCF for a UE) provides the PCF instance ID within the "pcfId"attribute, and optionally the recovery timestamp within the "recoveryTime" attribute, the BSF may use this information to carry out the clean-up procedures defined in subclause 6.4 of 3GPP TS 23.527 [17], if necessary.

Upon the reception of an HTTP POST request with: "{apiRoot}/nbsf-management/<apiVersion>/pcf-ue-bindings" as Resource URI and "PcfForUeBinding" data structure as request body, the BSF shall:

-	create new binding information;

-	assign a bindingId; and

-	store the binding information.

The PCF as a NF service consumer may provide information about the PCF Set and the binding level of subsequent request to the same or different PCF instances for the Npcf_AMPolicyControl service. The PCF may provide the PCF Set Id within the "pcfSetId" attribute and "bindLevel" attribute set to NF_SET, or may provide the PCF Set Id within the "pcfSetId" attribute, PCF instance Id within the "pcfId" attribute and "bindLevel" attribute set to NF_INSTANCE.

If the BSF created an "Individual PCF for a UE Binding" resource, the BSF shall respond with "201 Created" status code with the message body containing a representation of the created binding information, as shown in figure 4.2.2.3-1, step 2. The BSF shall include a Location HTTP header field containing the URI of the created binding information, i.e. "{apiRoot}/nbsf-management/<apiVersion>/pcf-ue-bindings/{bindingId}".

If errors occur when processing the HTTP POST request, the PCF shall apply error handling procedures as specified in clause 5.7.

# *** Next Changes ***

#### 5.6.2.10	Type PcfForUeBinding

Table 5.6.2.10-1: Definition of type PcfForUeBinding

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supi | Supi | M | 1 | Subscription Permanent Identifier |  |
| gpsi | Gpsi | O | 0..1 | Generic Public Subscription Identifier |  |
| pcfForUeFqdn | Fqdn | C | 0..1 | FQDN of the PCF hosting the Npcf_AMPolicyAuthorization service, if available. (NOTE) |  |
| pcfForUeIpEndPoints | array(IpEndPoint) | C | 1..N | IP end points of the PCF hosting the Npcf_AMPolicyAuthorization service, if available. (NOTE) |  |
| pcfId | NfInstanceId | O | 0..1 | PCF instance identifier |  |
| recoveryTime | DateTime | O | 0..1 | Contains the recovery timestamp of the NF service consumer (e.g., PCF for the UE). This attribute may be present only in the HTTP POST request. | Recovery |
| pcfSetId | NfSetId | O | 0..1 | The PCF set Id |  |
| bindLevel | BindingLevel | O | 0..1 | Contains the level of binding. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Used to negotiate the supported optional features as described in clause 5.8. Shall be present in the HTTP POST request/response or in the HTTP GET response if the "supp-feat" query parameter is included in the HTTP GET request. |  |
| NOTE: At least one of the "pcfForUeFqdn" attribute or "pcfForUeIpEndPoints" attribute shall be provided. |  |  |  |  |  |

# *** Next Changes ***

## 5.8	Feature negotiation

The optional features in table 5.8-1 are defined for the Nbsf_Management Service API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [6].

Table 5.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | MultiUeAddr | This feature indicates the support of multiple UE addresses (IPv6 prefixes or MAC addresses) in the same binding information. |
| 2 | BindingUpdate | The consumer can use this feature for updating the session binding information. |
| 3 | SamePcf | This feature indicates the support of same PCF selection for the indicated combination. (NOTE) |
| 4 | ES3XX | Extended Support for 3xx redirections. This feature indicates the support of redirection for any service operation, according to Stateless NF procedures as specified in clauses 6.5.3.2 and 6.5.3.3 of 3GPP TS 29.500 [6] and according to HTTP redirection principles for indirect communication, as specified in clause 6.10.9 of 3GPP TS 29.500 [6]. |
| 5 | ExtendedSamePcf | This feature extends the support of same PCF selection for the indicated combination. This feature requires the support of SamePcf feature. (NOTE) |
| 6 | AddSnssaiDnnPair | This feature indicates the support of additional S-NSSAI and DNN pair(s) for which the binding event report(s) apply. |
| 7 | Recovery | This feature indicates the support of providing the recovery time of the NF service consumer within the PCF for the UE bindling. |
| NOTE: The "SamePcf" feature is applicable to the deployments where the N5 and/or Rx interface apply and the UE address is available in the PCF at the creation of the SM Policy Association. The "ExtendedSamePcf" feature is applicable for any PCF deployment, regardless of UE address availability at the creation of SM Policy association and/or N5 and/or Rx applicability. |  |  |

# *** Next Changes ***

# A.2	Nbsf_Management API

openapi: 3.0.0

info:

version: 1.5.0-alpha.3

title: Nbsf_Management

description: |

Binding Support Management Service API.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.521 V19.4.0; 5G System; Binding Support Management Service.

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.521/'

servers:

- url: '{apiRoot}/nbsf-management/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

security:

- {}

- oAuth2ClientCredentials:

- nbsf-management

paths:

/pcfBindings:

post:

summary: Create a new Individual PCF for a PDU Session binding information

operationId: CreatePCFBinding

tags:

- PCF Bindings (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PcfBinding'

responses:

'201':

description: The creation of an individual PCF for a PDU Session binding.

content:

application/json:

schema:

$ref: '#/components/schemas/PcfBinding'

headers:

Location:

description: >

Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

description: >

The existing PCF binding information stored in the BSF for the indicated combination is

returned.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ExtProblemDetails'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

get:

summary: Read PCF for a PDU Session Bindings information

operationId: GetPCFBindings

tags:

- PCF Bindings (Collection)

parameters:

- name: ipv4Addr

in: query

description: The IPv4 Address of the served UE.

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

- name: ipv6Prefix

in: query

description: >

The IPv6 Address of the served UE. The NF service consumer shall append '/128' to the

IPv6 address in the attribute value. E.g. '2001:db8:85a3::8a2e:370:7334/128'.

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

- name: macAddr48

in: query

description: The MAC Address of the served UE.

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MacAddr48'

- name: dnn

in: query

description: DNN.

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

- name: supi

in: query

description: Subscription Permanent Identifier.

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: gpsi

in: query

description: Generic Public Subscription Identifier

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

- name: snssai

in: query

description: The identification of slice.

required: false

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

- name: ipDomain

in: query

description: The IPv4 address domain identifier.

required: false

schema:

type: string

- name: supp-feat

in: query

description: To filter irrelevant responses related to unsupported features.

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: >

The individual PCF for a PDU Session binding session binding information resource

matching the query parameter(s) is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PcfBinding'

'204':

description: >

There is no PCF for a PDU Session binding information matching the query parameter(s).

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/pcfBindings/{bindingId}:

delete:

summary: Delete an existing Individual PCF for a PDU Session Binding information

operationId: DeleteIndPCFBinding

tags:

- Individual PCF Binding (Document)

parameters:

- name: bindingId

in: path

description: Represents the individual PCF for a PDU Session Binding.

required: true

schema:

type: string

responses:

'204':

description: >

No Content. The Individual PCF for a PDU Session Binding information resource is

deleted.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Update an existing Individual PCF for a PDU Session Binding information

operationId: UpdateIndPCFBinding

tags:

- Individual PCF for a PDU Session Binding (Document)

parameters:

- name: bindingId

in: path

description: Represents the individual PCF for a PDU Session Binding.

required: true

schema:

type: string

requestBody:

description: Parameters to update the existing PCF for a PDU Session binding.

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/PcfBindingPatch'

responses:

'200':

description: OK (Successful update of the PCF for a PDU Session binding).

content:

application/json:

schema:

$ref: '#/components/schemas/PcfBinding'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions:

post:

operationId: CreateIndividualSubcription

summary: Create an individual subscription for event notifications from the BSF

tags:

- Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/BsfSubscription'

responses:

'201':

description: Created.

headers:

Location:

description: >

Contains the URI of the newly created resource.

required: true

schema:

type: string

content:

application/json:

schema:

$ref: '#/components/schemas/BsfSubscriptionResp'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

myNotification:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/BsfNotification'

responses:

'204':

description: No Content. Notification was successful.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subId}:

put:

operationId: ReplaceIndividualSubcription

summary: Replace an individual subscription for event notifications from the BSF

tags:

- IndividualSubscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/BsfSubscription'

parameters:

- name: subId

in: path

description: Subscription correlation ID

required: true

schema:

type: string

responses:

'200':

description: OK. Resource was successfully modified and representation is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/BsfSubscriptionResp'

'204':

description: No Content. Resource was successfully modified.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

operationId: DeleteIndividualSubcription

summary: Delete an individual subscription for event notifications from the BSF

tags:

- IndividualSubscription (Document)

parameters:

- name: subId

in: path

description: Subscription correlation ID

required: true

schema:

type: string

responses:

'204':

description: No Content. Resource was successfully deleted.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/pcf-ue-bindings:

post:

summary: Create a new Individual PCF for a UE binding information

operationId: CreatePCFforUEBinding

tags:

- PCF for a UE Bindings (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PcfForUeBinding'

responses:

'201':

description: The creation of an individual PCF for a UE binding.

content:

application/json:

schema:

$ref: '#/components/schemas/PcfForUeBinding'

headers:

Location:

description: >

Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

get:

summary: Read PCF for a UE Bindings information

operationId: GetPCFForUeBindings

tags:

- PCF for a UE Bindings (Collection)

parameters:

- name: supi

in: query

description: Subscription Permanent Identifier.

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: gpsi

in: query

description: Generic Public Subscription Identifier

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

- name: supp-feat

in: query

description: To filter irrelevant responses related to unsupported features.

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: >

The individual PCF for a UE binding session binding information resource matching the

query parameter(s) is returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/PcfForUeBinding'

minItems: 0

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/pcf-ue-bindings/{bindingId}:

delete:

summary: Delete an existing Individual PCF for a UE Binding information

operationId: DeleteIndPCFforUEBinding

tags:

- Individual PCF for a UE Binding (Document)

parameters:

- name: bindingId

in: path

description: Represents the individual PCF for a UE Binding.

required: true

schema:

type: string

responses:

'204':

description: >

No Content. The Individual PCF for a UE binding information resource is deleted.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Update an existing Individual PCF for a UE Binding information

operationId: UpdateIndPCFforUEBinding

tags:

- Individual PCF for a UE Binding (Document)

parameters:

- name: bindingId

in: path

description: Represents the individual PCF for a UE Binding.

required: true

schema:

type: string

requestBody:

description: Parameters to update the existing PCF for a UE binding.

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/PcfForUeBindingPatch'

responses:

'200':

description: OK. Successful update of the PCF for a PDU Session binding.

content:

application/json:

schema:

$ref: '#/components/schemas/PcfForUeBinding'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/pcf-mbs-bindings:

post:

summary: Create a new Individual PCF for an MBS Session binding.

operationId: CreatePCFMbsBinding

tags:

- PCF for an MBS Session Bindings (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PcfMbsBinding'

responses:

'201':

description: >

Created. A new Individual PCF for an MBS Session Binding resource is created

and the corresponding URI is returned in an HTTP Location header.

content:

application/json:

schema:

$ref: '#/components/schemas/PcfMbsBinding'

headers:

Location:

description: >

Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

description: >

The existing PCF binding information stored in the BSF for the MBS session is

returned.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/MbsExtProblemDetails'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

get:

summary: Retrieve an existing PCF for an MBS Session binding.

operationId: GetPCFMbsBinding

tags:

- PCF for an MBS Session Bindings (Collection)

parameters:

- name: mbs-session-id

in: query

description: >

Contains the identifier of the MBS Session to which the requested MBS Session

binding is related.

required: true

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsSessionId'

- name: supp-feat

in: query

description: >

Contains the list of features supported by the NF service consumer and used to

filter irrelevant responses related to unsupported features.

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: >

OK. The Individual PCF for an MBS Session Binding resource(s) matching the provided

query parameter(s) are returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/PcfMbsBinding'

minItems: 0

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/pcf-mbs-bindings/{bindingId}:

parameters:

- name: bindingId

in: path

description: >

Represents the identifier of the Individual PCF for an MBS Session Binding resource.

required: true

schema:

type: string

patch:

summary: Request the modification of an existing Individual PCF for an MBS Session Binding resource.

operationId: ModifyIndPCFMbsBinding

tags:

- Individual PCF for an MBS Session Binding (Document)

requestBody:

description: Parameters to request the modification of the PCF for an MBS Session Binding.

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/PcfMbsBindingPatch'

responses:

'200':

description: >

OK. The Individual PCF for an MBS Session Binding resource is successfully modified and

a representation of the updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/PcfMbsBinding'

'204':

description: >

No Content. The Individual PCF for an MBS Session Binding resource is successfully

modified and no content is returned in the response body.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual PCF for an MBS Session Binding.

operationId: DeleteIndPCFMbsBinding

tags:

- Individual PCF for an MBS Session Binding (Document)

responses:

'204':

description: >

No Content. The Individual PCF for an MBS Session Binding resource is successfully

Deleted.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nbsf-management: Access to the Nbsf_Management API

schemas:

PcfBinding:

description: Identifies an Individual PCF for a PDU Session binding.

type: object

properties:

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

ipv4Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

ipv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

addIpv6Prefixes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

minItems: 1

description: The additional IPv6 Address Prefixes of the served UE.

ipDomain:

type: string

macAddr48:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MacAddr48'

addMacAddrs:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MacAddr48'

minItems: 1

description: The additional MAC Addresses of the served UE.

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

pcfFqdn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

pcfIpEndPoints:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/IpEndPoint'

minItems: 1

description: IP end points of the PCF hosting the Npcf_PolicyAuthorization service

pcfDiamHost:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DiameterIdentity'

pcfDiamRealm:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DiameterIdentity'

pcfSmFqdn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

pcfSmIpEndPoints:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/IpEndPoint'

minItems: 1

description: IP end points of the PCF hosting the Npcf_SMPolicyControl service.

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

pcfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

pcfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

recoveryTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

paraCom:

$ref: '#/components/schemas/ParameterCombination'

bindLevel:

$ref: '#/components/schemas/BindingLevel'

ipv4FrameRouteList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4AddrMask'

minItems: 1

ipv6FrameRouteList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

minItems: 1

required:

- dnn

- snssai

PcfBindingPatch:

description: Identifies an Individual PCF binding used in an HTTP Patch method.

type: object

properties:

ipv4Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4AddrRm'

ipDomain:

type: string

nullable: true

ipv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6PrefixRm'

addIpv6Prefixes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

minItems: 1

description: The additional IPv6 Address Prefixes of the served UE.

nullable: true

macAddr48:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MacAddr48Rm'

addMacAddrs:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MacAddr48'

minItems: 1

description: The additional MAC Addresses of the served UE.

nullable: true

pcfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

pcfFqdn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

pcfIpEndPoints:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/IpEndPoint'

minItems: 1

description: IP end points of the PCF hosting the Npcf_PolicyAuthorization service.

pcfDiamHost:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DiameterIdentity'

pcfDiamRealm:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DiameterIdentity'

ParameterCombination:

description: >

Represents the combination used by the BSF to check whether there is an existing PCF binding

information.

type: object

properties:

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

ExtProblemDetails:

description: >

Contains the FQDN or IP endpoints of the existing PCF and the cause value if there is an

existing PCF binding information for the indicated combination.

allOf:

- $ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

- $ref: '#/components/schemas/BindingResp'

MbsExtProblemDetails:

description: >

Contains the FQDN or IP endpoints of the existing PCF and the cause value if there is an

existing PCF binding information for the MBS session.

allOf:

- $ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

- $ref: '#/components/schemas/MbsBindingResp'

BindingResp:

description: Contains the binding information for a PCF for a PDU Session.

type: object

properties:

pcfSmFqdn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

pcfSmIpEndPoints:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/IpEndPoint'

minItems: 1

description: IP end points of the PCF hosting the Npcf_SMPolicyControl service.

MbsBindingResp:

description: Contains the binding information for a PCF for an MBS Session.

type: object

properties:

pcfFqdn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

pcfIpEndPoints:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/IpEndPoint'

minItems: 1

description: IP end points of the PCF handling the MBS Session.

anyOf:

- required: [pcfFqdn]

- required: [pcfIpEndPoints]

BsfSubscription:

description: Contains the event subscription data.

type: object

properties:

events:

type: array

items:

$ref: '#/components/schemas/BsfEvent'

minItems: 1

description: Contain te subscribed events.

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifCorreId:

type: string

description: Notification Correlation ID assigned by the NF service consumer.

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

snssaiDnnPairs:

$ref: '#/components/schemas/SnssaiDnnPair'

addSnssaiDnnPairs:

type: array

items:

$ref: '#/components/schemas/SnssaiDnnPair'

minItems: 1

description: >

Represents the additional S-NSSAI and DNN pair(s) for which the binding event report(s)

shall apply.

expiry:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- events

- notifUri

- notifCorreId

- supi

BsfNotification:

description: Contains the event notifications.

type: object

properties:

notifCorreId:

type: string

eventNotifs:

type: array

items:

$ref: '#/components/schemas/BsfEventNotification'

minItems: 1

description: Notifications about Individual Events.

required:

- notifCorreId

- eventNotifs

BsfEventNotification:

description: Contains an event notification.

type: object

properties:

event:

$ref: '#/components/schemas/BsfEvent'

pcfForUeInfo:

$ref: '#/components/schemas/PcfForUeInfo'

pcfForPduSessInfos:

type: array

items:

$ref: '#/components/schemas/PcfForPduSessionInfo'

minItems: 1

description: The information of the PCF for a PDU session.

matchSnssaiDnns:

type: array

items:

$ref: '#/components/schemas/SnssaiDnnPair'

minItems: 1

description: Matching S-NSSAI and DNN pairs.

required:

- event

PcfForUeInfo:

description: Contains the information of the PCF for a UE.

type: object

properties:

pcfFqdn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

pcfIpEndPoints:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/IpEndPoint'

minItems: 1

description: IP end points of the PCF hosting the Npcf_AmPolicyAuthorization service.

pcfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

pcfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

bindLevel:

$ref: '#/components/schemas/BindingLevel'

PcfForPduSessionInfo:

description: Contains the informaiton of the PCF for a PDU session.

type: object

properties:

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

pcfFqdn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

pcfIpEndPoints:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/IpEndPoint'

minItems: 1

description: IP end points of the PCF hosting the Npcf_AmPolicyAuthorization service.

ipv4Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4AddrRm'

ipDomain:

type: string

ipv6Prefixes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

minItems: 1

description: The IPv6 Address Prefixes of the served UE.

macAddrs:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MacAddr48'

minItems: 1

description: The MAC Addresses of the served UE.

pcfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

pcfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

bindLevel:

$ref: '#/components/schemas/BindingLevel'

required:

- snssai

- dnn

PcfForUeBinding:

description: Identifies an Individual PCF for a UE binding.

type: object

properties:

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

pcfForUeFqdn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

pcfForUeIpEndPoints:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/IpEndPoint'

minItems: 1

description: IP end points of the PCF hosting the Npcf_AmPolicyAuthorization service.

pcfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

recoveryTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

pcfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

bindLevel:

$ref: '#/components/schemas/BindingLevel'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- supi

anyOf:

- required: [pcfForUeFqdn]

- required: [pcfForUeIpEndPoints]

PcfForUeBindingPatch:

description: Identifies the updates of an Individual PCF for a UE binding.

type: object

properties:

pcfForUeFqdn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

pcfForUeIpEndPoints:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/IpEndPoint'

minItems: 1

description: IP end points of the PCF hosting the Npcf_AmPolicyAuthorization service.

pcfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

SnssaiDnnPair:

description: Contains a S-NSSAI and DNN combination.

type: object

required:

- snssai

- dnn

properties:

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

PcfMbsBinding:

description: Represents an Individual PCF for an MBS Session binding.

type: object

properties:

mbsSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsSessionId'

pcfFqdn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

pcfIpEndPoints:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/IpEndPoint'

minItems: 1

pcfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

pcfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

bindLevel:

$ref: '#/components/schemas/BindingLevel'

recoveryTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- mbsSessionId

anyOf:

- required: [pcfFqdn]

- required: [pcfIpEndPoints]

PcfMbsBindingPatch:

description: >

Represents the requested modification to an Individual PCF for an MBS Session binding.

type: object

properties:

pcfFqdn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

pcfIpEndPoints:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/IpEndPoint'

minItems: 1

pcfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

BindingLevel:

anyOf:

- type: string

enum:

- NF_SET

- NF_INSTANCE

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents the binding level.

Possible values are:

- NF_SET: Indicates the NF set level of binding.

- NF_INSTANCE: Indicates the NF instance level of binding.

BsfEvent:

anyOf:

- type: string

enum:

- PCF_PDU_SESSION_BINDING_REGISTRATION

- PCF_PDU_SESSION_BINDING_DEREGISTRATION

- PCF_UE_BINDING_REGISTRATION

- PCF_UE_BINDING_DEREGISTRATION

- SNSSAI_DNN_BINDING_REGISTRATION

- SNSSAI_DNN_BINDING_DEREGISTRATION

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the event notified by the BSF.

Possible values are:

- PCF_PDU_SESSION_BINDING_REGISTRATION: Indicates the binding of a PCF for a PDU session

is registered.

- PCF_PDU_SESSION_BINDING_DEREGISTRATION: Indicates the binding of a PCF for a PDU session

is deregistered.

- PCF_UE_BINDING_REGISTRATION: Indicates the binding of a PCF for a UE is registered.

- PCF_UE_BINDING_DEREGISTRATION: Indicates the binding of a PCF for a UE is deregistered.

- SNSSAI_DNN_BINDING_REGISTRATION: Indicates the binding of a PCF for a PDU session

corresponding to the first PDU session for a DNN and S-NSSAI combination is registered.

- SNSSAI_DNN_BINDING_DEREGISTRATION: Indicates the binding of a PCF for a PDU session

corresponding to the first PDU session for a DNN and S-NSSAI combination is deregistered.

BsfSubscriptionResp:

description: >

It represents a response to a modification or creation request of an Individual Binding

Subscription resource. It may contain the notification of the already met events.

anyOf:

- $ref: '#/components/schemas/BsfSubscription'

- $ref: '#/components/schemas/BsfNotification'

# *** End of Changes ***