3GPP TSG-CT WG4 Meeting #131	C4-254391

Sophia-Antipolis; 13th – 17th October 2025	was C4-254383

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.502 | CR | 0911 | rev | 5 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Essential Clarification on AF Coordination Information Handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc19 |  |  |  |  |  | Date: |  |  | 2025-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As specified in 3GPP TS 23.502, when Change of SSC mode 3 PDU Session Anchor with multiple PDU Sessions is needed and coordination between 5GC and AF is enabled, the old SMF includes the old SM Context Ref with reference to the SM context including the AF Coordination Information in SM Context Status Notification; the AMF then subsequently provide the old SM Context Ref to the new SMF if a new SMF is selected. The new SMF when received such an old SM Context Ref shall retrieve the AF coordination Information from old SMF. In TS 29.502, the above procedure was implemented in different service operations without any description for the service logic between these steps, which makes the whole feature very difficult to understand or the reader. This CR adds essential clarifications to describe the relationship between the service operations according to the stage 2 requirements. This CR also removes one duplicated application error. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the service logic relations between the service operations for AF coordination information handling, according to stage 2 requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear description of the AF coordination handling in the specifications, may lead to interworking issues in multi-vendor deployments when runtime coordination between 5GC and AF is enabled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.2, 6.1.6.2.8, 6.1.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact any yaml files. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: - Add reference to the SM Context resource URI - Editorial corrections Rev2: - Clarify the corresponding feature in the condition of the description texts Rev3: - Add new rows for description texts, one row per applicable feature. Rev4: Editorial corrections Rev5: Update the identifier to URI in the description text. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.1.6.2.2	Type: SmContextCreateData

Table 6.1.6.2.2-1: Definition of type SmContextCreateData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ************* Table Rows skipped for clarity ************************ |  |  |  |  |  |
| oldSmContextRef | Uri | C | 0..1 | This IE shall be present if smfTransferInd is set to true. When present, this IE shall contain the URI of the SM Context resource in the old SMF. | CTXTR |
|  |  |  |  | This IE shall be present, without smfTransferInd set, if this information was received earlier with the resourceStatus IE set to "UNCHANGED" in Notify SM Context Status. When present, this IE shall contain the URI of the SM Context resource (with structure as specified in clause 6.1.3.3.2) in the old SMF including AF coordination information, see clause 4.3.5.2 of 3GPP TS 23.502 [3]. This IE shall be used by the SMF to retrieve the AF coordination information from the old SMF, as specified in clause 5.2.2.6.1. | EnEDGE |
| wAgfInfo | WAgfInfo | C | 0..1 | This IE shall be present, if received from the W-AGF. When present, it shall contain information about the N3 terminations of the W-AGF. The SMF may use this information when selecting the UPF. |  |
| tngfInfo | tngfInfo | C | 0..1 | This IE shall be present, if received from the TNGF. When present, it shall contain information about the N3 terminations of the TNGF. The SMF may use this information when selecting the UPF. |  |
| twifInfo | twifInfo | C | 0..1 | This IE shall be present, if received from the TWIF. When present, it shall contain information about the N3 terminations of the TWIF. The SMF may use this information when selecting the UPF. |  |
| ranUnchangedInd | boolean | C | 0..1 | This IE shall be present if the NG-RAN is not changed in case of I-SMF/V-SMF change or insertion during CM-CONNECTED registration procedure after EPS to 5GS handover (see clause 5.2.2.2.7) or I-SMF/V-SMF selection per DNAI (see clause 5.2.2.2.12). When present, it shall be set as follows: - true: NG-RAN is not changed; - false: NG-RAN is changed. | DTSSA |
| samePcfSelectionInd | boolean | C | 0..1 | This IE shall be present, if the AMF received the PCF Selection Assistance info from the UDM indicating that the same PCF is required. (NOTE 4) When present, it shall be set as follows: - True: the SMF is indicated to select the same PCF instance for the PDU session. - False (default): the SMF is not indicated to select the same PCF instance for the PDU session. |  |
| ************* Table Rows skipped for clarity ************************ |  |  |  |  |  |

* * * Next Change * * * *

##### 6.1.6.2.8	Type: SmContextStatusNotification

Table 6.1.6.2.8-1: Definition of type SmContextStatusNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ************* Table Rows skipped for clarity ************************ |  |  |  |  |  |
| oldSmContextRef | Uri | C | 0..1 | This IE may be present if resourceStatus in statusInfo is: - TRANSFERRED When present, this IE shall include the URI of the SM Context resource in the old I-SMF or SMF. | CTXTR |
|  |  |  |  | This IE may be present with the resourceStatus IE in statusInfo is set to "UNCHANGED", if the SMF selection during PDU Session re-establishment for SSC mode 3 is needed and the runtime coordination between old SMF and AF is enabled. When present, this IE shall contain the URI of the SM Context resource (with structure as specified in clause 6.1.3.3.2) in the old SMF including the AF coordination information, see clause 4.3.5.2 of 3GPP TS 23.502 [3]. | EnEDGE |
| altAnchorSmfUri | Uri | C | 0..1 | This IE shall be present if resourceStatus in statusInfo is: - ALT_ANCHOR_SMF When present, it shall contain the API URI of the alternative (H-)SMF towards which the PDU session is established. | AASN |
| altAnchorSmfId | NfInstanceId | C | 0..1 | This IE may be present if resourceStatus in statusInfo is: - ALT_ANCHOR_SMF When present, it shall contain the NF Instance Id of the alternative (H-)SMF towards which the PDU session is established. | AASN |
| targetDnaiInfo | TargetDnaiInfo | C | 0..1 | This IE shall be present if the I-SMF selection or removal for the current PDU session, or SMF selection during PDU Session re-establishment for SSC mode 2/3 is needed, or if it is received from the SMF in Notify Status. When present, this IE shall include the target DNAI Information. | EnEDGE |
| oldPduSessionRef | Uri | C | 0..1 | This IE shall be present if resourceStatus in statusInfo is "UNCHANGED", the SMF selection during PDU Session re-establishment for SSC mode 3 is needed and the runtime coordination between old SMF and AF is enabled. When present, this IE shall contain the URI of the PDU session resource in the old SMF. The URI shall be an absolute URI, including apiRoot (see clause 6.1.3.6.2). | EnEDGE |

* * * Next Change * * * *

#### 6.1.7.3	Application Errors

The common application errors defined in Table 5.2.7.2-1 of 3GPP TS 29.500 [4] may be used for the Nsmf_PDUSession service.

The following application errors listed in Table 6.1.7.3-1 are specific to the Nsmf_PDUSession service.

Table 6.1.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| N1_SM_ERROR | 403 Forbidden | This indicates that an error, other than those listed in this table, was detected when processing the N1 SM information received in the request, e.g. N1 SM protocol error, or a PDU session establishment is rejected due to Operator Determined Barring. |
| SNSSAI_DENIED | 403 Forbidden | The subscriber does not have the necessary subscription to access the SNSSAI. |
| DNN_DENIED | 403 Forbidden | The subscriber does not have the necessary subscription to access the DNN. |
| PDUTYPE_DENIED | 403 Forbidden | The subscriber does not have the necessary subscription for the requested PDU session type. |
| SSC_DENIED | 403 Forbidden | The subscriber does not have the necessary subscription for the requested SSC mode. |
| SUBSCRIPTION_DENIED | 403 Forbidden | This indicates an error, other than those listed in this table, due to lack of necessary subscription to serve the UE request. |
| DNN_NOT_SUPPORTED | 403 Forbidden | The DNN is not supported by the SMF. |
| PDUTYPE_NOT_SUPPORTED | 403 Forbidden | The requested PDU session type is not supported by the SMF for the PDN corresponding to the DNN. |
| SSC_NOT_SUPPORTED | 403 Forbidden | The requested SSC mode is not supported by the SMF for the PDN corresponding to the DNN. |
| HOME_ROUTED_ROAMING_REQUIRED | 403 Forbidden | It is used in LBO roaming, if the V-SMF is not able to process some part of the N1 SM information that requires Home Routed Roaming. |
| OUT_OF_LADN_SERVICE_AREA | 403 Forbidden | The PDU session corresponds to a LADN and the UE is outside of the LADN Service Area. |
| N2_SM_ERROR | 403 Forbidden | This indicates that an error, other than those listed in this table, was detected when processing the N2 SM information received in the request, e.g. N2 SM protocol error. |
| PRIORITIZED_SERVICES_ONLY | 403 Forbidden | The SMF was notified that the UE is reachable only for regulatory prioritized service and the PDU Session to be activated is not for a regulatory prioritized service. |
| PDU_SESSION_ANCHOR_CHANGE | 403 Forbidden | The SMF decided to change the PDU Session Anchor for the PDU Session. |
| TARGET_MME_CAPABILITY | 403 Forbidden | A request to retrieve an SM context is rejected due to the target MME not capable to support the PDU session. |
| NO_EPS_5GS_CONTINUITY | 403 Forbidden | It is used during an EPS to 5GS Idle mode mobility or handover, if the PDU session does not support seamless session continuity to 5GS. |
| UNABLE_TO_PAGE_UE | 403 Forbidden | The request is rejected due to a temporarily inability to page the UE. |
| UE_NOT_RESPONDING | 403 Forbidden | The UE did not respond to the request initiated by the network, e.g. paging. |
| REJECTED_BY_UE | 403 Forbidden | The request is rejected by the UE. |
| REJECTED_DUE_VPLMN_POLICY | 403 Forbidden | The request is rejected due to VPLMN operator policy. |
| HO_TAU_IN_PROGRESS | 403 Forbidden | The request is rejected temporarily due to a mobilty procedure in progress. |
| INTEGRITY_PROTECTED_MDR_NOT_ACCEPTABLE | 403 Forbidden | The integrity protected maximum data rate value provided by the UE is not acceptable for the PDU session based on local policy at the SMF. This error is applicable when the UP Security Policy for the PDU Session is determined to have Integrity Protection set to "Required". An NF service consumer that receives this error cause may use it for maintaining KPIs. |
| EBI_EXHAUSTED | 403 Forbidden | The allocation of EPS Bearer ID failed due to exhaustion of EBI as the maximum number of EBIs has already been allocated to the UE. |
| EBI_REJECTED_LOCAL_POLICY | 403 Forbidden | The allocation of EPS Bearer ID was rejected due to local policy in the Serving PLMN. |
| EBI_REJECTED_NO_N26 | 403 Forbidden | The allocation of EPS Bearer ID was rejected when the AMF is in a serving PLMN that does not support 5GS-EPS interworking procedures with N26 interface. |
| DEFAULT_EPS_BEARER_INACTIVE | 403 Forbidden | It is used during EPS to 5GS mobility if the default EPS bearer context of the PDU session is reported as inactive by the UE in the epsBearerCtxStatus attribute. |
| HANDOVER_RESOURCE_ALLOCATION_FAILURE | 403 Forbidden | It is used during a N2 handover preparation or an EPS to 5GS handover preparation, if no resource is allocated by the target NG-RAN for the PDU session. |
| LATE_OVERLAPPING_REQUEST | 403 Forbidden | The request is rejected because it collides with an existing SM context or PDU session context with a more recent origination timestamp (see clause 5.2.3.3). |
| DEFAULT_EBI_NOT_TRANSFERRED | 403 Forbidden | It is used during 5GS to EPS mobility if the EBI of the default EPS bearer is included in the notToTransferEbiList attribute. |
| NOT_SUPPORTED_WITH_ISMF | 403 Forbidden | The request is rejected due to a requested functionality that is not supported for a PDU session with an I-SMF/V-SMF. |
| SERVICE_NOT_AUTHORIZED_BY_NEXT_HOP | 403 Forbidden | The SMF is not authorized to access service provided by next hop NF producer, e.g. H-SMF or SMF or old I-SMF or old V-SMF. |
| NO_DATA_FORWARDING | 403 Forbidden | The request to setup data forwarding tunnels is rejected because none of the EPS bearer contexts received in the request body contains an F-TEID for DL data forwarding. |
| S_NSSAI_UNAVAILABLE_DUE_TO_NSAC | 403 Forbidden | The NSACF has returned error for the requested S-NSSAI and hence the PDU Session cannot be transferred from non-3gpp to 3gpp. |
| EXCEEDED_UE_SLICE_DATA_RATE | 403 Forbidden | The request is rejected due to the maximum bit rate per S-NSSAI per UE is exceeded, when the SMF receives the same application error from the PCF. |
| EXCEEDED_SLICE_DATA_RATE | 403 Forbidden | The request is rejected due to the maximum bit rate per S-NSSAI is exceeded, when the SMF receives the same application error from the PCF. |
| CONTEXT_NOT_FOUND | 404 Not Found | It is used when no context corresponding to the request exists in the SMF. |
| HIGHER_PRIORITY_REQUEST_ONGOING | 409 Conflict | The request is rejected temporarily due to procedure for higher priority session in progress. |
| UE_IN_CM_IDLE_STATE | 409 Conflict | The request is rejected due to the UE being in CM-IDLE state for the PDU session associated to non-3GPP access. |
| INSUFFICIENT_RESOURCES_SLICE | 500 Internal Server Error | The request cannot be provided due to insufficient resources for the specific slice. |
| INSUFFICIENT_RESOURCES_SLICE_DNN | 500 Internal Server Error | The request cannot be provided due to insufficient resources for the specific slice and DNN. |
| DNN_CONGESTION | 503 Service Unavailable | The SMF has detected congestion for the requested DNN and performs overload control for that DNN which does not allow the PDU session to be established. |
| S_NSSAI_CONGESTION | 503 Service Unavailable | During PDU session establishment, the SMF has detected congestion for the requested S-NSSAI (including the congestion due to NSAC failure) and performs overload control for that S-NSSAI which does not allow the PDU session to be established. During handover between 3GPP access and non-3GPP access, the SMF has detected congestion for the requested S-NSSAI on the target access (e.g., due to NSAC) and performs overload control for that S-NSSAI on the target access which does not allow the PDU session to be handover to the target access. |
| PEER_NOT_RESPONDING | 504 Gateway Timeout | No response is received from a remote peer, or the remote peer is known to be not reachable, e.g. to indicate that no response has been received from the H-SMF for a HR PDU session or the SMF for a PDU session with I-SMF. |
| NETWORK_FAILURE | 504 Gateway Timeout | The request is rejected due to a network problem. |
| UPF_NOT_RESPONDING | 504 Gateway Timeout | The request is rejected due to no response received from the UPF. |
| UE_NOT_REACHABLE | 504 Gateway Timeout | The UE is not reachable for service. |

* * * End of Changes * * * *