CT WG3 Meeting #83	C3-155080

16 - 20 November 2015, Anaheim, USA	(revision of C3-15xxxx)

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.214 | CR | 0425 | rev | - | Current version: | 13.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Incorrect command name to request access network information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NWK-PL2IMS-CT,TEI13 |  |  |  |  |  |  | Date: |  | 2015-11-20 |
| Category: | D |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | AF can request access network information as part of AAR and STR commands. However instead of STR command, the "S3GPP TR" is mentioned in clause 4.4.6.7. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The command name for STR is corrected. One typo in the same clause is corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misinterpretation of the used command. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.6.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR … CR … |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 4.4.6.7	Reporting Access Network Information

If the AF requests the PCRF to report the access network information (e.g. user location and/or user timezone information), the AF shall subscribe to the "ACCESS_NETWORK_INFO_REPORT" within the Specific-Action AVP and shall include the required access network information within the Required-Access-Info AVP. The AF may request the PCRF to report the access network information in conjunction with providing the PCRF with the AF session information, refer to clause 4.4.1. Optionally, the AF may request the PCRF to report the access network information without providing service information (see clause A.10.2). In the latter case the AF establishes an Rx session for the AF session upon requesting the access network information from the PCRF with an AA-Request command, containing information required for the session binding in the Framed-IP-Address AVP, the Framed-Ipv6-Prefix AVP Subscription-Id AVP, the Called-Station-Id AVP and/or the IP-Domain-Id AVP.

The AF may also request the PCRF to report the access network information at Rx session termination. To do so, the AF shall include the required access network information within the Required-Access-Info AVP in the corresponding ST-Request.

When the PCRF receives a request to report the access network information from the AF in an AAR command or in an STR command triggered by the AF, if the PCRF determines that the access network does not support the access network information reporting based on the currently used IP-CAN type or the values of RAT-Type AVP and AN-Trusted AVP or the PCEF/BBERF does not support the access network information reporting based on the Supported-Feature AVP, the PCRF shall respond to AF with an AAA or STA command including the NetLoc-Access-Support AVP set to the value of 0 (NETLOC_ACCESS_NOT_SUPPORTED); otherwise, it shall immediately configure the PCEF or BBERF to provide such access network information.

When the PCRF then receives the access network information from the PCEF/BBERF, the PCRF shall provide the corresponding access network information to the AF within the 3GPP-User-Location-Info AVP (if available), TWAN-Identifier AVP (if available), User-Location-Info-Time AVP (if available), 3GPP-SGSN-MCC-MNC AVP (if location info is not available) and/or 3GPP-MS-TimeZone AVP in the RAR command if the Rx session is not being terminated or in the STA command if the Rx session is being terminated. If the information is provided in the RAR command, PCRF shall also provide the ACCESS_NETWORK_INFO_REPORT within Specific-Action AVP.

NOTE 1:	The PCRF receives the access network information from the PCEF/BBERF if it is requested by the AF previously or the IP-CAN bearer/IP-CAN session is terminated.

When the PCRF receives the NetLoc-Access-Support AVP set to the value of 0 (NETLOC_ACCESS_NOT_SUPPORTED) from the PCEF/BBERF, the PCRF shall send a RAR command including the Specific-Action AVP set to INDICATION_OF_ACCESS_NETWORK_INFO_REPORTING_FAILURE and the NetLoc-Access-Support AVP set to the value of 0 (NETLOC_ACCESS_NOT_SUPPORTED) if the AF requested the access network information in an AAR command or send an STA command including the NetLoc-Access-Support AVP set to the value of 0 (NETLOC_ACCESS_NOT_SUPPORTED) if the AF requested the access network information in an S3GPP TR command.

NOTE 2:	The 3GPP GPRS, 3GPP EPS and Trusted WLAN support access network information reporting in this release.

The PCRF shall not send an RAR command with the ACCESS_NETWORK_INFO_REPORT value within a Specific-Action AVP to report any subsequently received access network information to the AF, unless the AF sends a new request for access network information.

* * * End of Change * * * *