3GPP TSG-CT6 Meeting #88	C6-180170

# Kunming (P.R. of China), 17th-20th April 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.111 | CR | 0688 | rev | - | Current version: | 14.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Make Date and Time field conditional in Data Connection Status Change event in 31.111. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14 |  |  |  |  |  | Date: |  |  | 2018-04-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In M2M devices, Date and Time set once in the device might not persist after boot up as they might not button battery. At that point, the only way for this device to receive Date and Time would be via NITZ. However, if NITZ is not supported by the network, the device would never receive Date and Time. In that case, the ME shall still send a valid ENVELOPE to the UICC even if the Date and Time are not available. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Making Date and Time field conditional in the Data Connection Status Change event ENVELOPE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | M2M devices might not be receive the Data Connection Status Change event ENVELOPE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.25.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR xxxx |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 7.5.25.2 	Structure of ENVELOPE (EVENT DOWNLOAD – Data Connection Status Change)

Direction: ME to UICC

The command header is specified in TS 31.101 [13].

Command parameters/data

| Description | Clause | M/O/C | Min | Length |
| --- | --- | --- | --- | --- |
| Event download tag | 9.1 | M | Y | 1 |
| Length (A+B+C+D+E+F+G+H+I+J+K+L) | - | M | Y | 1 or 2 |
| Event list | 8.25 | M | Y | A |
| Device identities | 8.7 | M | Y | B |
| Data connection status | 8.137 | M | Y | D |
| Data connection type | 8.138 | M | Y | E |
| (E)SM cause | 8.139 | C | Y | F |
| Transaction identifier | 8.28 | M | Y | G |
| Date-Time and Time zone | 8.39 | C | Y | H |
| Location Information | 8.19 | C | Y | I |
| Access Technology | 8.62 | C | Y | J |
| Location status | 8.27 | M | Y | K |
| PDP context activation parameters or EPS PDN connection activation parameters | 8.72 or 8.98 | C | Y | L |

- 	Event list: the Event list data object shall contain only one event (value part of length 1 byte), and the ME shall set the event to:

•	Data Connection Status Change.

-	Device identities: the terminal shall set the device identities to:

•	source:		Network for network originated messages. ME for ME originated messages;

•	destination:	UICC.

-	Data connection status: This data object shall contain the status of the data connection.

-	(E)SM cause: If an (E)SM cause is available, this data object shall contain either the SM cause as defined in 3GPP TS 24.008 [9] or the ESM cause as defined in 3GPP TS 24.301 [46].

-	Transaction identifier: the Transaction identifier data object shall contain one transaction identifier as defined in clause 8.28.

-	Date-Time and Time zone: If the date-time and time zone information is available in the ME, this data object is mandatory and shall contain the Date-Time and Time zone at the ME detected moment of occurrence of the event.

-	Location Information: This data object contains the identification (MCC, MNC, LAC/TAC, Cell Identity) of the current serving cell of the UE. The comprehension required flag of this data object in this command shall be set to '0'. This data object shall be present if the data connection is performed over GERAN, UTRAN or E-UTRAN.

-	Access Technology: This data object shall contain the access technology of the rejecting or accepting network.

- 	Location Status: This data object indicates the current service state of the terminal.

- 	For a PDP context activation, the Activate PDP context request parameters shall be used, as defined in TS 24.008 [9].

-	For an EPS PDN connection activation, the PDN Connectivity Request parameters shall be used, as defined in TS 24.301 [46].

Response parameters/data: None for this type of ENVELOPE command.