3GPP TSG-CT WG1 Meeting #112bis	C1-186947

# Vilnius, Lithuania, 15-19 October 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.229 | CR | 6147 | rev | 3 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of the Location Source Parameter for the SIP Geolocation Header Field |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Deutsche Telekom, Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  |  | Date: |  | 2018-10-18 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are scenarios where a Geolocation header field can contain more than one location value. It can be beneficial for the recipient of this information, e.g. a PSAP, to know the identity that has set up the location value rather than solely relying on the order of the entries to decide which location value to consider first or which value is more trustworthy. This is achieved by the loc-src parameter for the Geolocation header field defined in draft-ietf-sipcore-locparam. |  |  |  |  |  |  |  |  |
| Summary of change: |  | draft-ietf-sipcore-locparam is added as a new reference. A procedure for the P-CSCF is added to remove a present loc-src parameter from a Geolocation header field received from a UE. This is in line with draft-ietf-sipcore-locparam. A procedure for the E-CSCF is added to include the loc-src parameter in the Geolocation header field. The loc-src parameter is included in the major capabilities table. Minor editorial errors are corrected as well. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A PSAP can not distinguish between location data set up by a UE and a network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.2.1, 5.11.3, A2.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | This CR creates a new IETF dependency on draft-ietf-sipcore-locparam. |  |  |  |  |  |  |  |  |

1st change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[1A]	3GPP TS 22.101: "Service aspects; Service principles".

[1B]	3GPP TS 22.003: "Circuit Teleservices supported by a Public Land Mobile Network (PLMN)".

[1C]	3GPP TS 22.011: "Service accessibility".

[2]	3GPP TS 23.002: "Network architecture".

[3]	3GPP TS 23.003: "Numbering, addressing and identification".

[4]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[4A]	3GPP TS 23.107: "Quality of Service (QoS) concept and architecture".

[4B]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[4C]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[4D]	3GPP TS 23.140 Release 6: "Multimedia Messaging Service (MMS); Functional description; Stage 2".

[5]	3GPP TS 23.218: "IP Multimedia (IM) Session Handling; IM call model".

[6]	3GPP TS 23.221: "Architectural requirements".

[7]	3GPP TS 23.228: "IP multimedia subsystem; Stage 2".

[7A]	3GPP TS 23.234: "3GPP system to Wireless Local Area Network (WLAN) interworking; System description".

[7B]	3GPP TS 23.401: "GPRS enhancements for E-UTRAN access".

[7C]	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) Centralized Services; Stage 2".

[7D]	3GPP TS 23.380: "IMS Restoration Procedures".

[7E]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[7F]	3GPP TS 23.334: "IMS Application Level Gateway (IMS-ALG) – IMS Access Gateway (IMS-AGW) interface".

[7G]	3GPP TS 24.103: "Telepresence using the IP Multimedia (IM) Core Network (CN) Subsystem (IMS); Stage 3".

[8]	3GPP TS 24.008: "Mobile radio interface layer 3 specification; Core Network protocols; Stage 3".

[8A]	3GPP TS 24.141: "Presence service using the IP Multimedia (IM) Core Network (CN) subsystem; Stage 3".

[8B]	3GPP TS 24.147: "Conferencing using the IP Multimedia (IM) Core Network (CN) subsystem; Stage 3".

[8C]	3GPP TS 24.234: "3GPP System to Wireless Local Area Network (WLAN) interworking; WLAN User Equipment (WLAN UE) to network protocols; Stage 3".

[8D]	Void.

[8E]	3GPP TS 24.279: "Combining Circuit Switched (CS) and IP Multimedia Subsystem (IMS) services, stage 3, Release 7".

[8F]	3GPP TS 24.247: "Messaging service using the IP Multimedia (IM) Core Network (CN) subsystem; Stage 3".

[8G]	3GPP TS 24.167: "3GPP IMS Management Object (MO); Stage 3".

[8H]	3GPP TS 24.173: "IMS Multimedia telephony communication service and supplementary services; Stage 3".

[8I]	3GPP TS 24.606: "Message Waiting Indication (MWI) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[8J]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[8K]	3GPP TS 24.323: "3GPP IMS service level tracing management object (MO)".

[8L]	3GPP TS 24.341: "Support of SMS over IP networks; Stage 3".

[8M]	3GPP TS 24.237: "IP Multimedia Subsystem (IMS) Service Continuity; Stage 3".

[8N]	3GPP TS 24.647: "Advice Of Charge (AOC) using IP Multimedia (IM) Core Network (CN) subsystem".

[8O]	3GPP TS 24.292: "IP Multimedia (IM) Core Network (CN) subsystem Centralized Services (ICS); Stage 3".

[8P]	3GPP TS 24.623: "Extensible Markup Language (XML) Configuration Access Protocol (XCAP) over the Ut interface for Manipulating Supplementary Services".

[8Q]	3GPP TS 24.182: "IP Multimedia Subsystem (IMS) Customized Alerting Tones (CAT); Protocol specification".

[8R]	3GPP TS 24.183: "IP Multimedia Subsystem (IMS) Customized Ringing Signal (CRS); Protocol specification".

[8S]	3GPP TS 24.616: "Malicious Communication Identification (MCID) using IP Multimedia (IM) Core Network (CN) subsystem".

[8T]	3GPP TS 24.305: "Selective Disabling of 3GPP User Equipment Capabilities (SDoUE) Management Object (MO)".

[8U]	3GPP TS 24.302: "Access to the Evolved Packet Core (EPC) via non-3GPP access networks; Stage 3".

[8V]	3GPP TS 24.303: "Mobility management based on Dual-Stack Mobile IPv6".

[8W]	3GPP TS 24.390: "Unstructured Supplementary Service Data (USSD) using IP Multimedia (IM) Core Network (CN) subsystem IMS".

[8X]	3GPP TS 24.139: "3GPP System-Fixed Broadband Access Network Interworking; Stage 3".

[8Y]	3GPP TS 24.322: "UE access to IMS services via restrictive access networks - stage 3".

[8Z]	3GPP TS 24.371: "Web Real Time Communication (WebRTC) Access to IMS".

[8ZA]	3GPP TS 24.525: "Business trunking; Architecture and functional description".

[8ZB]	3GPP TS 24.244: "Wireless LAN control plane protocol for trusted WLAN access to EPC; Stage 3".

[8ZC]	3GPP TS 24.337: "IP Multimedia (IM) Core Network (CN) subsystem IP Multimedia Subsystem (IMS) inter-UE transfer; Stage 3".

[8ZD]	3GPP TS 24.334: "Proximity-services (ProSe) User Equipment (UE) to Proximity-services (ProSe) Function Protocol aspects; Stage 3".

[8ZE]	3GPP TS 24.379: "Mission Critical Push To Talk (MCPTT) call control; Stage 3".

[8ZF]	3GPP TS 24.628: "Common Basic Communication procedures using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[8ZG]	3GPP TS 24.604: "Communication Diversion (CDIV) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[9]	3GPP TS 25.304: "User Equipment (UE) procedures in idle mode and procedures for cell reselection in connected mode".

[9A]	3GPP TS 25.331: "Radio Resource Control (RRC); Protocol Specification".

[9B]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[9C]	3GPP TS 26.267: "eCall Data Transfer; In-band modem solution; General description".

[10]	Void.

[10A]	3GPP TS 27.060: "Mobile Station (MS) supporting Packet Switched Services".

[11]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting Packet Based Services and Packet Data Networks (PDN)".

[11A]	3GPP TS 29.162: "Interworking between the IM CN subsystem and IP networks".

[11B]	3GPP TS 29.163: "Interworking between the IP Multimedia (IM) Core Network (CN) subsystem and Circuit Switched (CS) networks".

[11C]	3GPP TS 29.161: "Interworking between the Public Land Mobile Network (PLMN) supporting Packet Based Services with Wireless Local Access and Packet Data Networks (PDN)"

[11D]	3GPP TS 29.079: "Optimal Media Routeing within the IP Multimedia Subsystem".

[12]	3GPP TS 29.207 Release 6: "Policy control over Go interface".

[12A]	3GPP TS 29.273: "Evolved Packet System (EPS); 3GPP EPS AAA interfaces".

[13]	Void.

[13A]	3GPP TS 29.209 Release 6: "Policy control over Gq interface".

[13B]	3GPP TS 29.212: "Policy and Charging Control (PCC); Reference points".

[13C]	3GPP TS 29.213: "Policy and charging control signalling flows and Quality of Service (QoS) parameter mapping".

[13D]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point".

[14]	3GPP TS 29.228: "IP Multimedia (IM) Subsystem Cx and Dx Interfaces; Signalling flows and message contents".

[15]	3GPP TS 29.229: "Cx and Dx Interfaces based on the Diameter protocol, Protocol details".

[15A]	3GPP TS 29.311: "Service Level Interworking for Messaging Services".

[15B]	3GPP TS 31.103: "Characteristics of the IP multimedia services identity module (ISIM) application".

[15C]	3GPP TS 31.102: "Characteristics of the Universal Subscriber Identity Module (USIM) application".

[15D]	3GPP TS 31.111: "Universal Subscriber Identity Module (USIM) Application Toolkit (USAT)".

[16]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[17]	3GPP TS 32.260: "Telecommunication management; Charging management; IP Multimedia Subsystem (IMS) charging".

[17A]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[18]	3GPP TS 33.102: "3G Security; Security architecture".

[19]	3GPP TS 33.203: "Access security for IP based services".

[19A]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[19B]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[19C]	3GPP TS 33.328: "IP Multimedia Subsystem (IMS) media plane security".

[19D]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[19E]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[19F]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[20]	3GPP TS 44.018: "Mobile radio interface layer 3 specification; Radio Resource Control (RRC) protocol".

[20A]	RFC 2401 (November 1998): "Security Architecture for the Internet Protocol".

[20B]	RFC 1594 (March 1994): "FYI on Questions and Answers to Commonly asked "New Internet User" Questions".

[20C]	Void.

[20D]	Void.

[20E]	RFC 2462 (November 1998): "IPv6 Stateless Address Autoconfiguration".

[20F]	RFC 2132 (March 1997): "DHCP Options and BOOTP Vendor Extensions".

[20G]	RFC 2234 (November 1997): "Augmented BNF for Syntax Specification: ABNF".

[21]	RFC 2617 (June 1999): "HTTP Authentication: Basic and Digest Access Authentication".

[22]	RFC 3966 (December 2004): "The tel URI for Telephone Numbers".

[23]	RFC 4733 (December 2006): "RTP Payload for DTMF Digits, Telephony Tones and Telephony Signals".

[24]	RFC 6116 (March 2011): "The E.164 to Uniform Resource Identifiers (URI) Dynamic Delegation Discovery System (DDDS) Application (ENUM)".

[25]	RFC 6086 (October 2009): "Session Initiation Protocol (SIP) INFO Method and Package Framework".

[25A]	RFC 3041 (January 2001): "Privacy Extensions for Stateless Address Autoconfiguration in IPv6".

[26]	RFC 3261 (June 2002): "SIP: Session Initiation Protocol".

[27]	RFC 3262 (June 2002): "Reliability of provisional responses in Session Initiation Protocol (SIP)".

[27A]	RFC 3263 (June 2002): "Session Initiation Protocol (SIP): Locating SIP Servers".

[27B]	RFC 3264 (June 2002): "An Offer/Answer Model with Session Description Protocol (SDP)".

[28]	RFC 6665 (July 2012): "SIP Specific Event Notification".

[28A]	Void.

[29]	RFC 3311 (September 2002): "The Session Initiation Protocol (SIP) UPDATE method".

[30]	RFC 3312 (October 2002): "Integration of resource management and Session Initiation Protocol (SIP)".

[31]	RFC 3313 (January 2003): "Private Session Initiation Protocol (SIP) Extensions for Media Authorization".

[32]	RFC 3320 (March 2002): "Signaling Compression (SigComp)".

[33]	RFC 3323 (November 2002): "A Privacy Mechanism for the Session Initiation Protocol (SIP)".

[34]	RFC 3325 (November 2002): "Private Extensions to the Session Initiation Protocol (SIP) for Network Asserted Identity within Trusted Networks".

[34A]	RFC 3326 (December 2002): "The Reason Header Field for the Session Initiation Protocol (SIP)".

[35]	RFC 3327 (December 2002): "Session Initiation Protocol Extension Header Field for Registering Non-Adjacent Contacts".

[35A]	RFC 3361 (August 2002): "Dynamic Host Configuration Protocol (DHCP-for-IPv4) Option for Session Initiation Protocol (SIP) Servers".

[36]	RFC 3515 (April 2003): "The Session Initiation Protocol (SIP) REFER method".

[37]	RFC 3420 (November 2002): "Internet Media Type message/sipfrag".

[37A]	RFC 3605 (October 2003): "Real Time Control Protocol (RTCP) attribute in Session Description Protocol (SDP)".

[38]	RFC 3608 (October 2003): "Session Initiation Protocol (SIP) Extension Header Field for Service Route Discovery During Registration".

[39]	RFC 4566 (June 2006): "SDP: Session Description Protocol".

[40]	RFC 3315 (July 2003): "Dynamic Host Configuration Protocol for IPv6 (DHCPv6)".

[40A]	RFC 2131 (March 1997): "Dynamic host configuration protocol".

[41]	RFC 3319 (July 2003): "Dynamic Host Configuration Protocol (DHCPv6) Options for Session Initiation Protocol (SIP) Servers".

[42]	RFC 3485 (February 2003): "The Session Initiation Protocol (SIP) and Session Description Protocol (SDP) static dictionary for Signaling Compression (SigComp)".

[43]	RFC 3680 (March 2004): "A Session Initiation Protocol (SIP) Event Package for Registrations".

[44]	Void.

[45]	Void.

[46]	Void.

[47]	Void.

[48]	RFC 3329 (January 2003): "Security Mechanism Agreement for the Session Initiation Protocol (SIP)".

[49]	RFC 3310 (September 2002): "Hypertext Transfer Protocol (HTTP) Digest Authentication Using Authentication and Key Agreement (AKA)".

[50]	RFC 3428 (December 2002): "Session Initiation Protocol (SIP) Extension for Instant Messaging".

[51]	Void.

[52]	RFC 7315 (July 2014): "Private Header (P-Header) Extensions to the Session Initiation Protocol (SIP) for the 3GPP".

[52A]	RFC 7976 (September 2016): "Updates to Private Header (P-Header) Extension Usage in Session Initiation Protocol (SIP) Requests and Responses".

[52B]	draft-jesske-update-p-visited-network-00 (July 2017): "Update to Private Header Field P-Visited-Network-ID in Session Initiation Protocol (SIP) Requests and Responses".

Editor's note (WI: IMSProtoc8, CR#5979): The above document cannot be formally referenced until it is published as an RFC.

[53]	RFC 3388 (December 2002): "Grouping of Media Lines in Session Description Protocol".

[54]	RFC 3524 (April 2003): "Mapping of Media Streams to Resource Reservation Flows".

[55]	RFC 3486 (February 2003): "Compressing the Session Initiation Protocol (SIP)".

[55A]	RFC 3551 (July 2003): "RTP Profile for Audio and Video Conferences with Minimal Control".

[56]	RFC 3556 (July 2003): "Session Description Protocol (SDP) Bandwidth Modifiers for RTP Control Protocol (RTCP) Bandwidth".

[56A]	RFC 3581 (August 2003): "An Extension to the Session Initiation Protocol (SIP) for Symmetric Response Routing".

[56B]	RFC 3841 (August 2004): "Caller Preferences for the Session Initiation Protocol (SIP)".

[56C]	RFC 3646 (December 2003): "DNS Configuration options for Dynamic Host Configuration Protocol for IPv6 (DHCPv6)".

[57]	ITU-T Recommendation E.164: "The international public telecommunication numbering plan".

[58]	RFC 4028 (April 2005): "Session Timers in the Session Initiation Protocol (SIP)".

[59]	RFC 3892 (September 2004): "The Session Initiation Protocol (SIP) Referred-By Mechanism".

[60]	RFC 3891 (September 2004): "The Session Inititation Protocol (SIP) "Replaces" Header".

[61]	RFC 3911 (October 2004): "The Session Inititation Protocol (SIP) "Join" Header".

[62]	RFC 3840 (August 2004): "Indicating User Agent Capabilities in the Session Initiation Protocol (SIP)".

[63]	RFC 3861 (August 2004): "Address Resolution for Instant Messaging and Presence".

[63A]	RFC 3948 (January 2005): "UDP Encapsulation of IPsec ESP Packets".

[64]	RFC 4032 (March 2005): "Update to the Session Initiation Protocol (SIP) Preconditions Framework".

[65]	RFC 3842 (August 2004) "A Message Summary and Message Waiting Indication Event Package for the Session Initiation Protocol (SIP)"

[65A]	RFC 4077 (May 2005): "A Negative Acknowledgement Mechanism for Signaling Compression".

[66]	RFC 7044 (February 2014): "An Extension to the Session Initiation Protocol (SIP) for Request History Information".

[67]	RFC 5079 (December 2007): "Rejecting Anonymous Requests in the Session Initiation Protocol (SIP)".

[68]	RFC 4458 (January 2006): "Session Initiation Protocol (SIP) URIs for Applications such as Voicemail and Interactive Voice Response (IVR)".

[69]	RFC 5031 (January 2008): "A Uniform Resource Name (URN) for Emergency and Other Well-Known Services".

[70]	RFC 3903 (October 2004): "An Event State Publication Extension to the Session Initiation Protocol (SIP)".

[71]	Void.

[72]	RFC 3857 (August 2004): "A Watcher Information Event Template Package for the Session Initiation Protocol (SIP)".

[74]	RFC 3856 (August 2004): "A Presence Event Package for the Session Initiation Protocol (SIP)".

[74A]	RFC 3603 (October 2003): "Private Session Initiation Protocol (SIP) Proxy-to-Proxy Extensions for Supporting the PacketCable Distributed Call Signaling Architecture".

[74B]	RFC 3959 (December 2004): "The Early Session Disposition Type for the Session Initiation Protocol (SIP)".

[75]	RFC 4662 (August 2006): "A Session Initiation Protocol (SIP) Event Notification Extension for Resource Lists".

[77]	RFC 5875 (May 2010): "An Extensible Markup Language (XML) Configuration Access Protocol (XCAP) Diff Event Package".

[78]	RFC 4575 (August 2006): "A Session Initiation Protocol (SIP) Event Package for Conference State".

[79]	RFC 5049 (December 2007): "Applying Signaling Compression (SigComp) to the Session Initiation Protocol (SIP)".

[80]	Void.

[81]	Void.

[82]	RFC 4457 (April 2006): "The Session Initiation Protocol (SIP) P-User-Database Private-Header (P-header)".

[83]	RFC 4145 (September 2005): "TCP-Based Media Transport in the Session Description Protocol (SDP)".

[84]	RFC 4320 (January 2006): "Actions Addressing Identified Issues with the Session Initiation Protocol's (SIP) Non-INVITE Transaction".

[85]	3GPP2 C.S0005-D (March 2004): "Upper Layer (Layer 3) Signaling Standard for cdma2000 Standards for Spread Spectrum Systems".

[86]	3GPP2 C.S0024-B v3.0 (September 2009): "cdma2000 High Rate Packet Data Air Interface Standard".

[86A]	3GPP2 C.S0084-000 (April 2007): "Overview for Ultra Mobile Broadband (UMB) Air Interface Specification".

[86B]	3GPP2 X.S0060-0 v1.0: "HRPD Support for Emergency Services".

[86C]	3GPP2 X.S0057-B v2.0: "E-UTRAN - eHRPD Connectivity and Interworking: Core Network Aspects".

[86D]	3GPP2 C.S0014-C v1.0: "Enhanced Variable Rate Codec, Speech Service Options 3, 68, and 70 for Wideband Spread Spectrum Digital Systems".

[86E]	3GPP2 X.S0059-200-A v1.0: "cdma2000 Femtocell Network: 1x and IMS Network Aspects".

[86F]	3GPP2 S.R0048-A v4.0: "3G Mobile Equipment Identifier (MEID) - Stage 1".

[87]	ITU-T Recommendation J.112, "Transmission Systems for Interactive Cable Television Services"

[88]	PacketCable Release 2 Technical Report, PacketCable™ Architecture Framework Technical Report, PKT-TR-ARCH-FRM.

[89]	RFC 6442 (December 2011): "Location Conveyance for the Session Initiation Protocol".

[90]	RFC 4119 (December 2005) "A Presence-based GEOPRIV Location Object Format".

[91]	RFC 5012 (January 2008): "Requirements for Emergency Context Resolution with Internet Technologies".

[91A]	Void.

[92]	RFC 5626 (October 2009): "Managing Client Initiated Connections in the Session Initiation Protocol (SIP)".

[93]	RFC 5627 (October 2009): "Obtaining and Using Globally Routable User Agent URIs (GRUUs) in the Session Initiation Protocol (SIP)".

[94]	RFC 5628 (October 2009): "Registration Event Package Extension for Session Initiation Protocol (SIP) Globally Routable User Agent URIs (GRUUs)".

[95]	Void.

[96]	RFC 4168 (October 2005): "The Stream Control Transmission Protocol (SCTP) as a Transport for the Session Initiation Protocol (SIP)".

[97]	RFC 5002 (August 2007): "The Session Initiation Protocol (SIP) P-Profile-Key Private Header (P-Header)".

[98]	ETSI ES 283 035: "Telecommunications and Internet Converged Services and Protocols for Advanced Networks (TISPAN); Network Attachment Sub-System (NASS); e2 interface based on the DIAMETER protocol".

[99]	RFC 5245 (April 2010): "Interactive Connectivity Establishment (ICE): A Protocol for Network Address Translator (NAT) Traversal for Offer/Answer Protocols".

[100]	RFC 5389 (October 2008): "Session Traversal Utilities for NAT (STUN)".

[101]	RFC 5766 (April 2010): "Traversal Using Relays around NAT (TURN): Relay Extensions to Session Traversal Utilities for NAT (STUN)".

[102]	RFC 5768 (April 2010): "Indicating Support for Interactive Connectivity Establishment (ICE) in the Session Initiation Protocol (SIP)".

[103]	RFC 4967 (July 2007): "Dial String Parameter for the Session Initiation Protocol Uniform Resource Identifier".

[104]	RFC 5365 (October 2008): "Multiple-Recipient MESSAGE Requests in the Session Initiation Protocol (SIP)".

[105]	RFC 5368 (October 2008): "Referring to Multiple Resources in the Session Initiation Protocol (SIP)".

[106]	RFC 5366 (October 2008): "Conference Establishment Using Request-Contained Lists in the Session Initiation Protocol (SIP)".

[107]	RFC 5367 (October 2008): "Subscriptions to Request-Contained Resource Lists in the Session Initiation Protocol (SIP)".

[108]	RFC 4583 (November 2006): "Session Description Protocol (SDP) Format for Binary Floor Control Protocol (BFCP) Streams".

[109]	RFC 5009 (September 2007): "Private Header (P-Header) Extension to the Session Initiation Protocol (SIP) for Authorization of Early Media".

[110]	RFC 4354 (January 2006): "A Session Initiation Protocol (SIP) Event Package and Data Format for Various Settings in Support for the Push-to-Talk over Cellular (PoC) Service".

[111]	RFC 4964 (September 2007): "The P-Answer-State Header Extension to the Session Initiation Protocol for the Open Mobile Alliance Push to Talk over Cellular".

[112]	RFC 4694 (October 2006): "Number Portability Parameters for the 'tel' URI".

[113]	Void.

[114]	RFC 4769 (November 2006): "IANA Registration for an Enumservice Containing Public Switched Telephone Network (PSTN) Signaling Information".

[115]	RFC 4411 (February 2006): "Extending the Session Initiation Protocol (SIP) Reason Header for Preemption Events".

[116]	RFC 4412 (February 2006): "Communications Resource Priority for the Session Initiation Protocol (SIP)".

[117]	RFC 5393 (December 2008): "Addressing an Amplification Vulnerability in Session Initiation Protocol (SIP) Forking Proxies".

[118]	RFC 4896 (June 2007): "Signaling Compression (SigComp) Corrections and ClarificationsImplementer's Guide for SigComp".

[119]	RFC 5112 (January 2008): "The Presence-Specific Static Dictionary for Signaling Compression (Sigcomp)".

[120]	RFC 5688 (January 2010): "A Session Initiation Protocol (SIP) Media Feature Tag for MIME Application Subtype".

[121]	RFC 6050 (November 2010): "A Session Initiation Protocol (SIP) Extension for the Identification of Services".

[122]	Void.

[123]	Void.

[124]	RFC 3986 (January 2005): "Uniform Resource Identifiers (URI): Generic Syntax".

[125]	RFC 5360 (October 2008): "A Framework for Consent-Based Communications in the Session Initiation Protocol (SIP)".

[126]	RFC 7433 (January 2015): "A Mechanism for Transporting User-to-User Call Control Information in SIP".

[126A]	RFC 7434 (January 2015): "Interworking ISDN Call Control User Information with SIP".

[127]	3GPP2 X.S0011-E: "cdma2000 Wireless IP Network Standard ".

[130]	RFC 6432 (November 2011): "Carrying Q.850 Codes in Reason Header Fields in SIP (Session Initiation Protocol) Responses".

[131]	RFC 6544 (March 2012): "TCP Candidates with Interactive Connectivity Establishment (ICE)".

[132]	RFC 3023 (January 2001): "XML Media Types".

[133]	RFC 5502 (April 2009): "The SIP P-Served-User Private-Header (P-Header) for the 3GPP IP Multimedia (IM) Core Network (CN) Subsystem".

[134]	RFC 7316 (July 2014): "The Session Initiation Protocol (SIP) P-Private-Network-Indication PrivateHeader (P-Header)".

[135]	RFC 4585 (July 2006): "Extended RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/AVPF)".

[136]	RFC 5104 (February 2008): "Codec Control Messages in the RTP Audio-Visual Profile with Feedback (AVPF)".

[137]	RFC 5939 (September 2010): "Session Description Protocol (SDP) Capability Negotiation".

[138]	ETSI ES 282 001: "Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); NGN Functional Architecture Release 1".

[139]	Void.

[140]	draft-ietf-insipid-logme-marking-07 (May 2017): "Marking SIP Messages to be Logged".

Editor's note (WI: ISAT, CR#5927): The above document cannot be formally referenced until it is published as an RFC.

[141]	Void.

[142]	RFC 6228 (May 2011): "Response Code for Indication of Terminated Dialog".

[143]	RFC 6223 (April 2011): "Indication of support for keep-alive".

[144]	RFC 4240 (December 2005): "Basic Network Media Services with SIP".

[145]	RFC 5552 (May 2009): "SIP Interface to VoiceXML Media Services".

[146]	RFC 6230 (May 2011): "Media Control Channel Framework".

[147]	RFC 6231 (May 2011): "An Interactive Voice Response (IVR) Control Package for the Media Control Channel Framework".

[148]	RFC 6505 (March 2012): "A Mixer Control Package for the Media Control Channel Framework".

[149]	RFC 2046 (November 1996): "Multipurpose Internet Mail Extensions (MIME) Part Two: Media Types".

[150]	RFC 5621 (September 2009): "Message Body Handling in the Session Initiation Protocol (SIP)".

[151]	RFC 3862 (August 2004): "Common Presence and Instant Messaging (CPIM): Message Format".

[152]	RFC 3890 (September 2004): "A Transport Independent Bandwidth Modifier for the Session Description Protocol (SDP)".

[153]	RFC 7254 (May 2014): "A Uniform Resource Name Namespace for the Global System for Mobile Communications Association (GSMA) and the International Mobile station Equipment Identity (IMEI)".

[154]	RFC 4122 (July 2005): "A Universally Unique IDentifier (UUID) URN Namespace".

[155]	RFC 7195 (May 2014): "Session Description Protocol (SDP) Extension for Setting Audio Media Streams over Circuit-Switched Bearers in the Public Switched Telephone Network (PSTN)".

[156]	RFC 7006 (September 2013): "Miscellaneous Capabilities Negotiation in the Session Description Protocol (SDP)".

[157]	RFC 5438 (January 2009): "Instant Message Disposition Notification (IMDN)".

[158]	RFC 5373 (November 2008): "Requesting Answering Modes for the Session Initiation Protocol (SIP)".

[160]	Void.

[161]	RFC 4288 (December 2005): "Media Type Specifications and Registration Procedures".

[162]	RFC 7989 (October 2016): "End-to-End Session Identification in IP-Based Multimedia Communication Networks".

[163]	RFC 6026 (September 2010): "Correct Transaction Handling for 2xx Responses to Session Initiation Protocol (SIP) INVITE Requests".

[164]	RFC 5658 (October 2009): "Addressing Record-Route issues in the Session Initiation Protocol (SIP)".

[165]	RFC 5954 (August 2010): "Essential Correction for IPv6 ABNF and URI Comparison in RFC3261".

[166]	RFC 4117 (June 2005): "Transcoding Services Invocation in the Session Initiation Protocol (SIP) using Third Party Call Control (3pcc)".

[167]	RFC 4567 (July 2006): "Key Management Extensions for Session Description Protocol (SDP) and Real Time Streaming Protocol (RTSP)".

[168]	RFC 4568 (July 2006): "Session Description Protocol (SDP) Security Descriptions for Media Streams".

[169]	RFC 3711 (March 2004): "The Secure Real-time Transport Protocol (SRTP)".

[170]	RFC 6043 (March 2011): "MIKEY-TICKET: Ticket-Based Modes of Key Distribution in Multimedia Internet KEYing (MIKEY)".

[171]	RFC 4235 (November 2005): "An INVITE-Initiated Dialog Event Package for the Session Initiation Protocol (SIP)".

[172]	RFC 6871 (February 2013): "SDP media capabilities Negotiation".

[173]	RFC 4488 (May 2006): "Suppression of Session Initiation Protocol (SIP) REFER Method Implicit Subscription".

[174]	Void.

[175]	RFC 7462 (March 2015): "URNs for the Alert-Info Header Field of the Session Initiation Protocol (SIP)".

[176]	ANSI/J-STD-036-B: "Enhanced Wireless 9-1-1, Phase 2".

[177]	Void.

[178]	RFC 4975 (September 2007): "The Message Session Relay Protocol (MSRP)".

[179]	RFC 3859 (August 2004): "Common Profile for Presence (CPP)".

[180]	RFC 3860 (August 2004): "Common Profile for Instant Messaging (CPIM)".

[181]	RFC 2368 (July 1998): "The mailto URL scheme".

[182]	RFC 4745 (February 2007): "Common Policy: A Document Format for Expressing Privacy Preferences".

[183]	RFC 5318 (December 2008): "The Session Initiation Protocol (SIP) P-Refused-URI-List Private-Header (P-Header)".

[184]	RFC 4538 (June 2006): "Request Authorization through Dialog Identification in the Session Initiation Protocol (SIP)".

[185]	RFC 5547 (May 2009): "A Session Description Protocol (SDP) Offer/Answer Mechanism to Enable File Transfer".

[186]	RFC 4483 (May 2006): "A Mechanism for Content Indirection in Session Initiation Protocol (SIP) Messages".

[187]	draft-atarius-dispatch-meid-urn-12 (May 2017): "A Uniform Resource Name Namespace for the Device Identity and the Mobile Equipment Identity (MEID)".

Editor's note: The above document cannot be formally referenced until it is published as an RFC.

[188]	RFC 6679 (August 2012): "Explicit Congestion Notification (ECN) for RTP over UDP".

[189]	RFC 3168 (September 2001): "The Addition of Explicit Congestion Notification (ECN) to IP".

[190]	RFC 6809 (November 2012): "Mechanism to Indicate Support of Features and Capabilities in the Session Initiation Protocol (SIP)".

[191]	RFC 6140 (March 2011): "Registration for Multiple Phone Numbers in the Session Initiation Protocol (SIP)".

[192]	RFC 6917 (April 2013): "Media Resource Brokering".

[193]	ETSI TS 101 454-1 v1.1.1: "Digital Video Broadcasting (DVB); Second Generation DVB Interactive Satellite System (DVB-RCS2); Part 1: Overview and System Level specification".

[194]	ETSI EN 301 545-2 v1.1.1: "Digital Video Broadcasting (DVB); Second Generation DVB Interactive Satellite System (DVB-RCS2); Part 2: Lower Layers for Satellite standard".

[195]	ETSI TS 101 545-3 v1.1.1: "Digital Video Broadcasting (DVB); Second Generation DVB Interactive Satellite System (DVB-RCS2); Part 3: Higher Layers Satellite Specification".

[196]	RFC 2616 (June 1999): "Hypertext Transfer Protocol -- HTTP/1.1".

[197]	RFC 7135 (May 2014): "IANA Registering a SIP Resource Priority Header Field Namespace for Local Emergency Communications".

[198]	RFC 6357 (August 2011): "Design Considerations for Session Initiation Protocol (SIP) Overload Control".

[199]	RFC 7339 (September 2014): "Session Initiation Protocol (SIP) Overload Control".

[200]	RFC 7415 (February 2015): "Session Initiation Protocol (SIP) Rate Control".

[201]	RFC 7200 (April 2014): "A Session Initiation Protocol (SIP) Load-Control Event Package".

[202]	ITU-T Recommendation T.38 (September 2010): "Procedures for real-time Group 3 facsimile communication over IP networks".

[203]	ISO 8601 (December 2004): "Date elements and interchange formats – Information interchange – Representation of dates and times".

[204]	RFC 5506 (April 2009): "Support for Reduced-Size Real-Time Transport Control Protocol (RTCP)".

[205]	RFC 3611 (November 2003): "RTP Control Protocol Extended Reports (RTCP XR)".

[206]	RFC 4796 (February 2007): "The Session Description Protocol (SDP) Content Attribute".

[207]	ISO 3166-1 (2006): "Codes for the representation of names of countries and their subdivisions – Part 1: Country codes".

[208]	RFC 8055 (January 2017): "Session Initiation Protocol (SIP) Via Header Field Parameter to Indicate Received Realm".

[209]	RFC 7090 (April 2014): "Public Safety Answering Point (PSAP) Callback".

[210]	RFC 5285 (July 2008): "A General Mechanism for RTP Header Extensions".

[211]	RFC 6236 (May 2011): "Negotiation of Generic Image Attributes in the Session Description Protocol (SDP)".

[212]	RFC 20 (May 2011): "ASCII format for Network Interchange".

[213]	RFC 5280 (May 2008): "Internet X.509 Public Key Infrastructure Certificate and Certificate Revocation List (CRL) Profile".

[214]	RFC 6714 (August 2012): "Connection Establishment for Media Anchoring (CEMA) for the Message Session Relay Protocol (MSRP)".

[215]	RFC 6135 (February 2011): "An Alternative Connection Model for the Message Session Relay Protocol (MSRP)".

[216]	Void.

[217]	RFC 7345 (August 2014): "UDP Transport Layer (UDPTL) over Datagram Transport Layer Security (DTLS)".

[218]	RFC 4279 (December 2005): "Pre-Shared Key Ciphersuites for Transport Layer Security (TLS)".

[219]	draft-ietf-mmusic-sctp-sdp-26 (April 2017): "Session Description Protocol (SDP) Offer/Answer Procedures For Stream Control Transmission Protocol (SCTP) over Datagram Transport Layer Security (DTLS) Transport".

Editor's note (WI: eIMS_TELEP, CR#4856): The above document cannot be formally referenced until it is published as an RFC.

[220]	RFC 2817 (May 2000): "Upgrading to TLS Within HTTP/1.1".

[221]	RFC 6062 (November 2010): "Using Relays around NAT (TURN) Extensions for TCP Allocations".

[222]	RFC 5763 (May 2010): "Framework for Establishing a Secure Real-time Transport Protocol (SRTP) Security Context Using Datagram Transport Layer Security (DTLS)".

[223]	RFC 5764 (May 2010): " Datagram Transport Layer Security (DTLS) Extension to Establish Keys for the Secure Real-time Transport Protocol (SRTP)".

[224]	RFC 7675 (October 2015): "STUN Usage for Consent Freshness".

[225]	RFC 7549 (May 2015): "3GPP SIP URI Inter Operator Traffic Leg Parameter".

[226]	draft-jesske-sipcore-sip-tree-cap-indicators-00 (June 2018): "Internet Assigned Numbers Authority (IANA) Registration of the Session Initiation Protocol (SIP) Feature-Capability indicators".

Editor's note [WI: IMSProtoc6, CR#4895]: The above document cannot be formally referenced until it is published as an RFC.

[227]	RFC 4169 (November 2005): "Hypertext Transfer Protocol (HTTP) Digest Authentication Using Authentication and Key Agreement (AKA) Version-2".

[228]	RFC 6947 (May 2013): "The Session Description Protocol (SDP) Alternate Connectivity (ALTC) Attribute".

[229]	Void.

[230]	RFC 8119 (March 2017): "SIP "cause" URI Parameter for Service Number Translation".

[231]	RFC 7647 (September 2015): "Clarifications for the Use of REFER with RFC6665".

[232]	RFC 7614 (August 2015): "Explicit Subscriptions for the REFER Method".

[233]	RFC 7621 (August 2015): "A Clarification on the Use of Globally Routable User Agent URIs (GRUUs) in the Session Initiation Protocol (SIP) Event Notification Framework".

[234]	RFC 7913 (June 2016): "P-Access-Network-Info ABNF Update".

[235]	RFC 7519 (May 2015): "JSON Web Token (JWT)".

[236]	Void.

[237]	RFC 5761 (April 2010): "Multiplexing RTP Data and Control Packets on a Single Port".

[237A]	RFC 8035 (November 2016): "Session Description Protocol (SDP) Offer/Answer Clarifications for RTP/RTCP Multiplexing".

[238]	draft-ietf-mmusic-data-channel-sdpneg-11 (January 2017): "SDP-based Data Channel Negotiation".

Editor's note [WI: eWebRTCi_CT, CR#5553]: The above document cannot be formally referenced until it is published as an RFC.

[239]	draft-ietf-sipcore-originating-cdiv-parameter-02 (May 2018): "P-Served-User Header Field Parameter for Originating CDIV session case in Session Initiation Protocol (SIP)".

Editor's note [WI: IMSProtoc5, CR#5584]: The above document cannot be formally referenced until it is published as an RFC.

[240]	draft-ietf-mmusic-dtls-sdp-32 (October 2017): "Session Description Protocol (SDP) Offer/Answer Considerations for Datagram Transport Layer Security (DTLS) and Transport Layer Security (TLS)".

Editor's note [WI: eMEDIASEC-CT, CR#5617]: The above document cannot be formally referenced until it is published as an RFC.

[241]	RFC 8122 (March 2017): "Connection-Oriented Media Transport over the Transport Layer Security (TLS) Protocol in the Session Description Protocol (SDP)".[242]	RFC 3863 (August 2004): "Presence Information Data Format".

[243]	RFC 4661 (September 2006): "An Extensible Markup Language (XML) Based Format for Event Notification Filtering".

[244]	RFC 8147 (May 2017): "Next-Generation Pan-European eCall".

[245]	CEN EN 15722:2015 (April 2015): "Intelligent transport systems - ESafety - ECall minimum set of data".

[246]	draft-ietf-mmusic-mux-exclusive-11 (February 2017): "Indicating Exclusive Support of RTP/RTCP Multiplexing using SDP".

Editor's note [WI: TEI14, IMS-WebRTC, CR#5615]: The above document cannot be formally referenced until it is published as an RFC.

[247]	RFC 7303 (July 2014): "XML Media Types".

[248]	IEEE Std 802.11-2012: "IEEE Standard for Information technology - Telecommunications and information exchange between systems - Local and metropolitan area networks - Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[249]	draft-ietf-mmusic-sdp-simulcast-08 (March 2017): "Using Simulcast in SDP and RTP Sessions".

Editor's note [WI: MMCMH_Enh-CT, CR#5739]: The above document cannot be formally referenced until it is published as an RFC.

[250]	draft-ietf-mmusic-rid-10 (March 2017): "RTP Payload Format Restrictions".

Editor's note [WI: MMCMH_Enh-CT, CR#5739]: The above document cannot be formally referenced until it is published as an RFC.

[251]	RFC 7728 (February 2016): "RTP Stream Pause and Resume".

[252]	RFC 8224 (February 2018): "Authenticated Identity Management in the Session Initiation Protocol (SIP)".[253]	RFC 5279 (July 2008): "A Uniform Resource Name (URN) Namespace for the 3rd Generation Partnership Project (3GPP)".

[254]	RFC 8197 (July 2017): "A SIP Response Code for Unwanted Calls".

[255]	draft-ietf-sipcore-reason-q850-loc-04 (August 2018): "ISUP Cause Location Parameter for the SIP Reason Header Field".

Editor's note [WI: REAS_EXT, CR#5883]: The above document cannot be formally referenced until it is published as an RFC.

[256]	RFC 8262 (October 2017): "Content-ID Header Field in the Session Initiation Protocol (SIP)".

[257]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[258]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (5GS); Stage 3".

[259]	RFC 4715 (November 2006): "The Integrated Services Digital Network (ISDN) Subaddress Encoding Type for tel URI".

[260]	3GPP TS 38.304: " NR; User Equipment (UE) procedures in idle mode and in RRC Inactive state".

[261]	IETF draft-ietf-stir-passport-shaken-02 (March 2018): "PASSporT SHAKEN Extension (SHAKEN)".

Editor's note [WI: eSPECTRE, CR#6113]:	The above document cannot be formally referenced until it is published as an RFC.

[262]	RFC 8225 (February 2018): "PASSporT: Personal Assertion Token"

Editor's note [WI: eSPECTRE, CR#6160]:	The above document cannot be formally referenced until it is published as an RFC.

[263]	3GPP TS 24.502: " Access to the 3GPP 5G Core Network (5GCN) via Non-3GPP Access Networks (N3AN); Stage 3".

[264]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2".

[265]	IETF draft-ietf-stir-passport-divert-03 (July 2018): "PASSporT Extension for Diverted Calls".

Editor's note [WI: eSPECTRE, CR#6185]:	The above document cannot be formally referenced until it is published as an RFC.

[xxx]	draft-ietf-sipcore-locparam-00 (August 2018): "Location Source Parameter for the SIP Geolocation Header Field".

Editor's note [WI: IMSProtoc16, CR#6147]: The above document cannot be formally referenced until it is published as an RFC.

2nd change

### 5.2.1	General

Where the P CSCF provides emergency call support, the procedures of subclause 5.2.10 shall be applied first.

Subclause 5.2.2 through subclause 5.2.9 define P-CSCF procedures for SIP that do not relate to emergency. All SIP requests are first screened according to the procedures of subclause 5.2.10 to see if they do relate to an emergency.

For all SIP transactions identified:

-	as relating to an emergency; or

-	if priority is supported, as containing an authorised Resource-Priority header field or a temporarily authorised Resource-Priority header field, or, if such an option is supported, relating to a dialog which previously contained an authorised Resource-Priority header field;

the P-CSCF shall give priority over other transactions or dialogs. This allows special treatment of such transactions or dialogs. If the P-CSCF recognises the need for priority processing to a request or if the P-CSCF recognises the need to provide different priority processing than the one indicated by the originating UE, based on the information stored during registration, the P-CSCF may insert or modify Resource-Priority header in accordance with RFC 4412 [116].

NOTE 1:	The special treatment can include filtering, higher priority processing, routeing, call gapping. The exact meaning of priority is not defined further in this document, but is left to national regulation and network configuration.

The P-CSCF shall support the Path and Service-Route header fields.

NOTE 2:	The Path header field is only applicable to the REGISTER request and its 200 (OK) response. The Service-Route header field is only applicable to the 200 (OK) response of REGISTER request.

NOTE 3:	In subsequent procedures, the P-CSCF can address the needs of individual users (e.g. in support of attached enterprise networks or in support of priority mechanisms, from information saved during registration. In this release of the specification, no information is specified in the registration procedures to perform this, and therefore this information has to either be associated with the user at time of registration from configured information, or by a mechanism outside the scope of this release of the specification.

When the P-CSCF sends any request or response to the UE, before sending the message the P-CSCF shall:

-	remove the P-Charging-Function-Addresses and P-Charging-Vector header fields, if present.

When the P-CSCF receives any request or response from the UE, the P-CSCF:

1)	shall remove the P-Charging-Function-Addresses and P-Charging-Vector header fields, if present. Also, the P-CSCF shall ignore any data received in the P-Charging-Function-Addresses and P-Charging-Vector header fields; and

2)	may insert previously saved values into the P-Charging-Function-Addresses header field before forwarding the message;

NOTE 4:	When the P-CSCF is located in the visited network, then it will not receive the P-Charging-Function-Addresses header field from the S-CSCF, IBCF, or I-CSCF. Instead, the P-CSCF discovers charging function addresses by other means not specified in this document.

3)	shall remove the P-Access-Network-Info header field, if the request or the response include a P-Access-Network-Info header field with a "network-provided" parameter;

4)	may insert a P-Access-Network-Info header field where:

a)	if no mechanism exists to support the access technology for this UE, the "network-provided" parameter is included, and the access-type field is set to a preconfigured value;

b)	if NASS is used to support the access technology for this UE, the "network-provided" parameter is included, and the access-type field is set:

-	when xDSL is the IP-CAN, to one of "ADSL", "ADSL2", "ADSL2+", "RADSL", "SDSL", "HDSL", "HDSL2", "G.SHDSL", "VDSL", "IDSL", or "xDSL", and the "dsl-location" parameter is set with the value received in the Location-Information header field in the User-Data Answer command as specified in ETSI ES 283 035 [98];

NOTE 5:	xDSL is a general abbreviation for all types of Digital Subscriber Lines, and "xDSL" is a possible access-type value of the P-Access-Network-Info header field.

-	when Ethernet is the IP-CAN, to one of "IEEE-802.3", "IEEE-802.3a", "IEEE-802.3e", "IEEE-802.3i", "IEEE-802.3j", "IEEE-802.3u","IEEE-802.3ab"or "IEEE-802.3ae", "IEEE-802.3ak", "IEEE-802.3aq", "IEEE-802.3an", "IEEE-802.3y" or "IEEE-802.3z" and if NASS subsystem is used, and the "eth-location" parameter is set with the value received in the Location-Information header field in the User-Data Answer command as specified in ETSI ES 283 035 [98];

-	when Fiber is the IP-CAN, to one of "G-PON", "XGPON1" or "IEEE-802.3ah" and if NASS subsystem is used, and the "fiber-location" parameter is set with the value received in the Location-Information header field in the User-Data Answer command as specified in ETSI ES 283 035 [98];

c)	if the PCRF is used to support the access technology for this UE and 3GPP-User-Location-Info as specified in 3GPP TS 29.214 [13D] is not available:

-	if the IP-CAN-Type value provided by the PCRF is not "DVB-RCS2", then:

I)	the access-type field or the access-class field is set to a value consistent with that received from the PCRF in the IP-CAN-Type, RAT-Type and AN-Trusted parameters using the procedures specified in 3GPP TS 29.214 [13D].

If the IP-CAN-Type parameter is set "Non-3GPP-EPS (6)" as specified in 3GPP TS 29.212 [13B], the RAT-Type parameter is set to "VIRTUAL (1)" as specified in 3GPP TS 29.212 [13B] and the AN-Trusted parameter is set to "UNTRUSTED (1)" as specified in 3GPP TS 29.273 [12A], the P-CSCF shall include the access-class field set to "untrusted-non-3GPP-VIRTUAL-EPC".

II)	if a 3GPP-MS-TimeZone parameter is available from the PCRF, then the "local-time-zone" parameter and the "daylight-saving-time" parameter may also be added using this information;

III)	the "network-provided" parameter is added;

IV)	if a TWAN-Identifier as specified in 3GPP TS 29.214 [13D] is received from the PCRF, the received TWAN-Identifier contains the Circuit-ID and the associated "Relay Identity", the received TWAN-Identifier does not contain the "Civic Address Information" and the P-CSCF is able to deduce a Geographical Identifier from the Circuit-ID and the associated "Relay Identity", then, if required by local operator policy, the P-CSCF shall include an operator-specific-GI field. The P-CSCF can obtain a Geographical Identifier from the CLF by using the e2 interface (see ETSI ES 283 035 [98]);

NOTE 6:	ETSI ES 283 035 [98] Release 3 enables querying a CLF using the User-Data-Request command in which the Global-Access-Id AVP contains the Fixed-Access-ID AVP set using the Circuit-ID value as the Logical-Access-ID and the "Relay Identity" as the Relay-Agent to get a corresponding Geographical Identifier. If multiple CLFs are deployed, the P-CSCF can dertermine which CLF to query based on the CGI or the SAI values or can use a DIAMETER proxy if deployed.

Editor's note: [IMSProtoc7, CR 5385] clause "2 References" needs to be modified to reference the release 3 of ETSI ES 283 035 [98].

V)	if WLAN Location Information as specified in 3GPP TS 23.402 [7E] is received from the PCRF, the received WLAN Location Information contains the location identifier and the P-CSCF is able to deduce a Geographical Identifier from the WLAN Location Information, then, if required by local operator policy, the P-CSCF shall include an operator-specific-GI field; and

VI)	if:

A)	the access-class field of the P-Access-Network-Info header field is set to "untrusted-non-3GPP-VIRTUAL-EPC"; or

B)	the access-class field of the P-Access-Network-Info header field is set to "3GPP-WLAN" and the AN-Trusted parameter specified in 3GPP TS 29.273 [12A] is received from PCRF and is set to "UNTRUSTED (1)";

then:

A)	if a UE-Local-IP-Address parameter specified in 3GPP TS 29.212 [13B] is received from the PCRF and if required by local operator policy, P-CSCF shall also include in the P-Access-Network-Info header field a UE-local-IP-address parameter set to the UE local IP address in the UE-Local-IP-Address parameter received from PCRF;

B)	if a UDP-Source-Port parameter specified in 3GPP TS 29.212 [13B] is received from the PCRF and if required by local operator policy, the P-CSCF shall also include in the P-Access-Network-Info header field a UDP-source-port parameter set to the UDP port in the UDP-Source-Port parameter received from PCRF;

C)	if a TCP-Source-Port parameter specified in 3GPP TS 29.212 [13B] is received from the PCRF and if required by local operator policy, the P-CSCF shall also include in the P-Access-Network-Info header field a TCP-source-port parameter set to the TCP port in the TCP-Source-Port parameter received from PCRF; and

D)	if an AN-GW-Address parameter specified in 3GPP TS 29.212 [13B] is received from the PCRF and if required by local operator policy, the P-CSCF shall also include in the P-Access-Network-Info header field an ePDG-IP-address parameter set to the ePDG IP address in the ePDG-IP-Address parameter received from PCRF; and

-	if the IP-CAN-Type value provided by the PCRF is "DVB-RCS2", then the "network-provided" parameter is included, the access-type field is set to "DVB-RCS2", and the "dvb-rcs2-node-id" parameter is set with the value provided by the IP-CAN provider;

d)	if the PCRF is used to support the access technology for this UE and 3GPP-User-Location-Info as specified in 3GPP TS 29.214 [13D] is available;

I)	the access-type field or the access-class field is set to a value consistent with that received from the PCRF in the IP-CAN-Type and RAT-Type parameters;

II)	the access-info field is set to a value consistent with the information received from the PCRF in the 3GPP-User-Location-Info parameter;

III)	if a 3GPP-MS-TimeZone parameter is available from the PCRF, then the "local-time-zone" parameter and the "daylight-saving-time" parameter may also be added using this information;

IV)	the "network-provided" parameter is added; and

V)	if required by local operator policy and the P-CSCF is able to deduce a Geographical Identifier from the Cell Global Identity (CGI) or form the Service Area Identifier (SAI) received from the PCRF, the P-CSCF shall include an operator-specific-GI field. The P-CSCF can obtain a Geographical Identifier from the CLF by using the e2 interface (see ETSI ES 283 035 [98]);

NOTE 7:	ETSI ES 283 035 [98] Release 3 enables querying a CLF using the User-Data-Request command in which the Global-Access-Id AVP contains the 3GPP-User-Location-Info AVP with a CGI or a SAI value to get a corresponding Geographical Identifier. If multiple CLFs are deployed, the P-CSCF can dertermine which CLF to query based on the CGI or the SAI values or can use a DIAMETER proxy if deployed.

Editor's note: [IMSProtoc7, CR 5167] clause "2 References" needs to be modified to reference the release 3 of ETSI ES 283 035 [98].

e)	if DOCSIS is used, and proprietary means of obtaining a location are used, the access-type field is set to "DOCSIS" and the "network-provided" parameter is added; and

f)	if none of NASS, PCRF and DOCSIS are used to support the access technology for the UE and the IP-CAN is not provided by the packet switched domain of the PLMN of the P-CSCF:

I)	if the P-CSCF is unaware of the radio access technology used by the UE, the access-class field is set to "VIRTUAL-no-PS";

II)	if the P-CSCF is aware that the radio access technology used by the UE is specified by IEEE Std 802.11-2012 [248], the access-class field is set to "WLAN-no-PS"; and

III)	the "network-provided" parameter is added;

5)	shall remove all Feature-Caps header fields, if present, from a UE that is not considered as privileged sender;

6)	may insert a P-Visited-Network-ID header field (except ACK, BYE, CANCEL, NOTIFY, PRACK, INFO and UPDATE) according to RFC 7976 [52A] with the value:

I)	of a pre-provisioned string that identifies the network of the P-CSCF at the home network; or

II)	if the UE is roaming in deployments without IMS-level roaming interfaces according to 3GPP TS 23.228 [7], a string that identifies the visited network of the UE including an indication that the P-CSCF is located in the home network;

7)	may insert a P-Visited-Network-ID header field in 200 (OK) response to INVITE request and in 200 (OK) response to MESSAGE request according to draft-jesske-update-p-visited-network [52B] and

8)	if a Geolocation header field is received from the UE, shall remove any present loc-src parameter from the Geolocation header field.

When the P-CSCF receives any request or response containing the P-Media-Authorization header field, the P-CSCF shall remove the header field.

NOTE 8:	Depending on the security mechanism in use, the P-CSCF can integrity protect all SIP messages sent to the UE outside of the registration and authentication procedures by using a security association or TLS session. The P-CSCF will discard any SIP message that is not protected by using a security association or TLS session and is received outside of the registration and authentication procedures. The integrity and confidentiality protection and checking requirements on the P-CSCF within the registration and authentication procedures are defined in subclause 5.2.2.

With the exception of 305 (Use Proxy) responses, the P-CSCF shall not recurse on 3xx responses.

NOTE 9:	If the P-CSCF is connected to a PDF the requirements for this interconnection is specified in the Release 6 version of this specification.

The P-CSCF may add, remove, or modify, the P-Early-Media header field within forwarded SIP requests and responses according to procedures in RFC 5009 [109].

NOTE 10:	The P-CSCF can use the P-Early-Media header field for the gate control procedures, as described in 3GPP TS 29.214 [13D]. In the presence of early media for multiple dialogs due to forking, if the P-CSCF is able to identify the media associated with a dialog, (i.e., if symmetric RTP is used by the UE and the P-CSCF can use the remote SDP information to determine the source of the media) the P-CSCF can selectively open the gate corresponding to an authorized early media flow for the selected media.

When SIP digest without TLS is used, the P-CSCF shall discard any SIP messages received outside of the registration and authentication procedures that do not map to an existing IP association as defined in subclause 5.2.3.

In case a device performing address and/or port number conversions is provided by a NA(P)T or NA(P)T-PT controlled by the P-CSCF, the P-CSCF may need to modify the SIP contents according to the procedures described in annex F. In case a device performing address and/or port number conversions is provided by a NA(P)T or NA(P)T-PT not controlled by the P-CSCF, the P-CSCF may need to modify the SIP contents according to the procedures described in annex K if both a "reg-id" and "+sip.instance" header field parameters are present in the received Contact header field as described in RFC 5626 [92].

The P-CSCF shall support the provision of the user-related policies (e.g. consideration of the user as a privileged sender):

-	from the S-CSCF during registration; and

-	by local configuration.

For the same policy, the precedence between the locally configured policy and a policy received during registration shall be based on local operator policy.

For UE performing the functions of an external attached networks using static mode of operation, the P-CSCF will receive requests to establish a TLS session that are not accompanied by the associated procedures of subclause 5.2.2. The P-CSCF shall permit the establishment of such TLS sessions, but subsequent operations without the reception of a REGISTER request shall only be permitted if the P-CSCF is configured for such a UE performing the functions of an external attached network using static mode of operation. Where a REGISTER request is received from a UE, the P-CSCF shall process the REGISTER request as defined in subclause 5.2.2, and shall not provide special procedures for a UE performing the functions of an external attached network using static mode of operation for the duration of the registration.

NOTE 11:	For requests other than REGISTER received from UEs that are not configured in this manner, then the procedures of subclause 5.2.6.3.2A apply.

NOTE 12:	The P-CSCF does not subscribe to the reg event package for a UE performing the functions of an external attached network using static mode of operation.

When sending a failure response to any received request, depending on operator policy, the P-CSCF may insert a Response-Source header field with an "fe" header field parameter constructed with the URN namespace "urn:3gpp:fe", the fe-id part of the URN set to "p-cscf" and optionally an appropriate fe-param part of the URN set in accordance with subclause 7.2.17. A P-CSCF when sending a failure response will add in the URN the "side" header field parameter set to:

-	"orig" for a UE-originating case; and

-	"term" for a UE-terminating case.

3rd change

### 5.11.3	Use of an LRF

Where the network operator determines that an LRF is to be used, the E-CSCF shall route initial requests for a dialog and standalone requests that contain an emergency service URN, i.e. a service URN with a top-level service type of "sos" as specified in RFC 5031 [69], or an emergency number, to the LRF in accordance with the procedures of RFC 3261 [26].

NOTE 1:	The E-CSCF is by definition responsible for emergency service URNs and is therefore allowed to change the Request-URI of requests containing emergency service URNs when a 3xx or 416 response is received.

For the outgoing request, the E-CSCF shall:

1)	insert a type 3 "orig-ioi" header field parameter in the P-Charging-Vector header field. The E-CSCF shall set the type 3 "orig-ioi" header field parameter to a value that identifies the sending network of the request. The E-CSCF shall not include the type 3 "term-ioi" header field parameter; and

2)	perform step 11 of subclause 5.11.2 before sending the INVITE request to the LRF.

When the E-CSCF receives any 3xx response to such a request, the E-CSCF shall select a Contact header field URI from the 3xx response according to RFC 3261 [26] and continue processing the steps given in subclause 5.11.2 with the following additions:

a)	at step 6), if item a) applies, place the URI received in the selected Contact header field URI in the 3xx response in the topmost entry in the Route header field;

b)	at step 6), if item b) applies, replace the original Request-URI with the URI received in the selected Contact header field URI in the 3xx response;

c)	if the user did not request privacy or if national regulator policy applicable to emergency services does not require the user be allowed to request privacy, then if the selected Contact header field URI contains a P-Asserted-Identity header field encoded as a header field of the URI, replace all P-Asserted-Identity header fields in the original request with this value;

NOTE 2:	Such a P-Asserted-Identity header field contains a reference number which is used in the communication between the PSAP and LRF.

d)	if operator local policies allow insertion of UE location information and if the received 3xx response contains a message/external-body MIME type as specified in RFC 4483 [186] with "access-type" MIME type parameter containing "URL" and "URL" MIME type parameter containing an HTTP or HTTPS URI identifying a PIDF location object as defined in RFC 4119 [90], then the E-CSCF shall insert a Geolocation header field containing this PIDF location object by reference (see RFC 6442 [89]);

e)	if the location source parameter for the SIP Geolocation header field as defined in draft-ietf-sipcore-locparam [xxx] is supported, include a loc-src parameter in the Geolocation header field set to the domain name of the visited network; and

f)	if operator policies allow forming requests from a URI and if 3xx response is received, then follow the procedures of RFC 3261 [26] subclause 19.1.5 with the following additions and clarifications:

-	replacement or inclusion of any header field from the URI in the selected Contact header field is subject to operator policy; and

-	if operator policy allows any LRF to provide a location by value, and the URI in the selected Contact header field contains the "Geolocation" header field, a "Geolocation-Routing" header field and a header field with hname "body" with a value, replace the entire message body with value of the header field with hname "body" in the URI in the selected Contact header field, otherwise do not perform this replacement.

If no 1xx or 2xx response to the request is received from the addressed PSAP within an operator settable timeout, or a 4xx – 5xx response is received, and additional URI values were included in the Contact header field of the response, the E-CSCF shall use these values according to RFC 3261 [26] in new requests that are otherwise generated according to the rules specified above.

If no 1xx or 2xx response to the request is received from the addressed PSAP within an operator settable timeout, or a 4xx – 5xx response is received, and all URI values included in the Contact header field of the 3xx response have been attempted, the E-CSCF shall use a default URI value configured in the E-CSCF in a new request that is otherwise generated according to the rules specified above.

If a 6xx response to the request is received, the E-CSCF acts in accordance with RFC 3261 [26].

When the E-CSCF receives any response related to the above request containing a "term-ioi" header field parameter, the E-CSCF shall store the value of the received "term-ioi" header field parameter received in the P-Charging-Vector header field, if present, and remove all received "orig-ioi" and "term-ioi" header field parameters from the forwarded response.

NOTE 3:	Any received "term-ioi" header field parameter will be a type 3 IOI. The IOI identifies the sending network of the response message.

If no 3xx response to the request is received from the LRF within an operator settable timeout, the E-CSCF shall use a default URI value configured in the E-CSCF in a request that is otherwise generated according to the rules specified above.

4th change

### A.2.1.2	Major capabilities

Table A.4: Major capabilities

| Item | Does the implementation support |  | Reference | RFC status | Profile status |
| --- | --- | --- | --- | --- | --- |
|  | Capabilities within main protocol |  |  |  |  |
| 1 | client behaviour for registration? |  | [26] subclause 10.2 | o | c3 |
| 2 | registrar? |  | [26] subclause 10.3 | o | c4 |
| 2A | registration of multiple contacts for a single address of record |  | [26] 10.2.1.2, 16.6 | o | o |
| 2B | initiating a session? |  | [26] subclause 13 | o | o |
| 2C | initiating a session which require local and/or remote resource reservation? |  | [30] | o | c43 |
| 3 | client behaviour for INVITE requests? |  | [26] subclause 13.2 | c18 | c18 |
| 4 | server behaviour for INVITE requests? |  | [26] subclause 13.3 | c18 | c18 |
| 5 | session release? |  | [26] subclause 15.1 | c18 | c18 |
| 6 | timestamping of requests? |  | [26] subclause 8.2.6.1 | o | o |
| 7 | authentication between UA and UA? |  | [26] subclause 22.2 | c34 | c34 |
| 8 | authentication between UA and registrar? |  | [26] subclause 22.2 | o | c74 |
| 8A | authentication between UA and proxy? |  | [26] 20.28, 22.3 | o | c75 |
| 9 | server handling of merged requests due to forking? |  | [26] 8.2.2.2 | m | m |
| 10 | client handling of multiple responses due to forking? |  | [26] 13.2.2.4 | m | m |
| 11 | insertion of date in requests and responses? |  | [26] subclause 20.17 | o | o |
| 12 | downloading of alerting information? |  | [26] subclause 20.4 | o | o |
|  | Extensions |  |  |  |  |
| 13 | SIP INFO method and package framework? |  | [25] | o | c100 |
| 13A | legacy INFO usage? |  | [25] 2, 3 | o | c90 |
| 14 | reliability of provisional responses in SIP? |  | [27] | c19 | c44 |
| 15 | the REFER method? |  | [36] | o | c33 |
| 15A | clarifications for the use of REFER with RFC6665? |  | [231] | c121 | c121 |
| 15B | explicit subscriptions for the REFER method? |  | [232] | o | o |
| 16 | integration of resource management and SIP? |  | [30] [64] | c19 | c44 |
| 17 | the SIP UPDATE method? |  | [29] | c5 | c44 |
| 19 | SIP extensions for media authorization? |  | [31] | o | c14 |
| 20 | SIP specific event notification? |  | [28] | o | c13 |
| 22 | acting as the notifier of event information? |  | [28] | c2 | c15 |
| 22A | a clarification on the use of GRUUs in the SIP event notification framework? |  | [233] | c122 | c122 |
| 23 | acting as the subscriber to event information? |  | [28] | c2 | c16 |
| 24 | session initiation protocol extension header field for registering non-adjacent contacts? |  | [35] | o | c6 |
| 25 | private extensions to the Session Initiation Protocol (SIP) for network asserted identity within trusted networks? |  | [34] | o | m |
| 26 | a privacy mechanism for the Session Initiation Protocol (SIP)? |  | [33] | o | m |
| 26A | request of privacy by the inclusion of a Privacy header indicating any privacy option? |  | [33] | c9 | c11 |
| 26B | application of privacy based on the received Privacy header? |  | [33] | c9 | n/a |
| 26C | passing on of the Privacy header transparently? |  | [33] | c9 | c12 |
| 26D | application of the privacy option "header" such that those headers which cannot be completely expunged of identifying information without the assistance of intermediaries are obscured? |  | [33] 5.1 | c10 | c27 |
| 26E | application of the privacy option "session" such that anonymization for the session(s) initiated by this message occurs? |  | [33] 5.2 | c10 | c27 |
| 26F | application of the privacy option "user" such that user level privacy functions are provided by the network? |  | [33] 5.3 | c10 | c27 |
| 26G | application of the privacy option "id" such that privacy of the network asserted identity is provided by the network? |  | [34] 7 | c10 | n/a |
| 26H | application of the privacy option "history" such that privacy of the History-Info header is provided by the network? |  | [66] 7.2 | c37 | c37 |
| 27 | a messaging mechanism for the Session Initiation Protocol (SIP)? |  | [50] | o | c7 |
| 28 | session initiation protocol extension header field for service route discovery during registration? |  | [38] | o | c17 |
| 29 | compressing the session initiation protocol? |  | [55] | o | c8 |
| 30 | private header extensions to the session initiation protocol for the 3rd-Generation Partnership Project (3GPP)? |  | [52] | o | m |
| 30A | act as first entity within the trust domain for asserted identity? |  | [34] | c96 | c97 |
| 30B | act as entity within trust network that can route outside the trust network? |  | [34] | c96 | c97 |
| 30C | act as entity passing on identity transparently independent of trust domain? |  | [34] | c96 | c98 |
| 31 | the P-Associated-URI header extension? |  | [52] 4.1, [52A] 4 | c21 | c22 |
| 32 | the P-Called-Party-ID header extension? |  | [52] 4.2, [52A] 4 | c21 | c23 |
| 33 | the P-Visited-Network-ID header extension? |  | [52] 4.3, [52A] 4 | c21 | c24 |
| 34 | the P-Access-Network-Info header extension? |  | [52] 4.4, [52A] 4, [234] 2 | c21 | c25 |
| 35 | the P-Charging-Function-Addresses header extension? |  | [52] 4.5, [52A] 4 | c21 | c26 |
| 36 | the P-Charging-Vector header extension? |  | [52] 4.6, [52A] 4 | c21 | c26 |
| 37 | security mechanism agreement for the session initiation protocol? |  | [48] | o | c20 |
| 37A | mediasec header field parameter for marking security mechanisms related to media? |  | Subclause 7.2A.7 | n/a | c101 |
| 38 | the Reason header field for the session initiation protocol? |  | [34A] | o | c68 |
| 38A | carrying Q.850 codes in reason header fields in SIP (Session Initiation Protocol) responses? |  | [130] | o | c82 |
| 38B | the location parameter for the SIP Reason header field? |  | [255] | o | c131 |
| 39 | an extension to the session initiation protocol for symmetric response routeing? |  | [56A] | o | c62 |
| 40 | caller preferences for the session initiation protocol? |  | [56B] | C29 | c29 |
| 40A | the proxy-directive within caller-preferences? |  | [56B] 9.1 | o.5 | o.5 |
| 40B | the cancel-directive within caller-preferences? |  | [56B] 9.1 | o.5 | o.5 |
| 40C | the fork-directive within caller-preferences? |  | [56B] 9.1 | o.5 | o.5 |
| 40D | the recurse-directive within caller-preferences? |  | [56B] 9.1 | o.5 | o.5 |
| 40E | the parallel-directive within caller-preferences? |  | [56B] 9.1 | o.5 | o.5 |
| 40F | the queue-directive within caller-preferences? |  | [56B] 9.1 | o.5 | o.5 |
| 41 | an event state publication extension to the session initiation protocol? |  | [70] | o | c30 |
| 42 | SIP session timer? |  | [58] | c19 | c19 |
| 43 | the SIP Referred-By mechanism? |  | [59] | o | c33 |
| 44 | the Session Inititation Protocol (SIP) "Replaces" header? |  | [60] | c19 | c38 (note 1) |
| 45 | the Session Inititation Protocol (SIP) "Join" header? |  | [61] | c19 | c19 (note 1) |
| 46 | the callee capabilities? |  | [62] | o | c35 |
| 47 | an extension to the session initiation protocol for request history information? |  | [66] | o | o |
| 47A | application of the "mp" optional header field parameter? |  | [66] | o | o |
| 47B | application of the "rc" optional header field parameter? |  | [66] | o | o |
| 47C | application of the "np" optional header field parameter? |  | [66] | o | o |
| 48 | Rejecting anonymous requests in the session initiation protocol? |  | [67] | o | o |
| 49 | session initiation protocol URIs for applications such as voicemail and interactive voice response? |  | [68] | o | o |
| 49A | Session Initiation Protocol (SIP) cause URI parameter for service number translation? |  | [230] | c118 | c118 |
| 50 | Session Initiation Protocol's (SIP) non-INVITE transactions? |  | [84] | m | m |
| 51 | the P-User-Database private header extension? |  | [82] 4 | o | c94 |
| 52 | a uniform resource name for services? |  | [69] | n/a | c39 |
| 53 | obtaining and using GRUUs in the Session Initiation Protocol (SIP)? |  | [93] | o | c40 (note 2) |
| 55 | the Stream Control Transmission Protocol (SCTP) as a Transport for the Session Initiation Protocol (SIP)? |  | [96] | o | c42 |
| 56 | the SIP P-Profile-Key private header extension? |  | [97] | n/a | n/a |
| 57 | managing client initiated connections in SIP? |  | [92] | o | c45 |
| 58 | indicating support for interactive connectivity establishment in SIP? |  | [102] | o | c46 |
| 59 | multiple-recipient MESSAGE requests in the session initiation protocol? |  | [104] | c47 | c48 |
| 60 | SIP location conveyance? |  | [89] | o | c49 |
| 60A | the Location Source parameter for the SIP Geolocation header field? |  | [xxx] | o | c133 |
| 61 | referring to multiple resources in the session initiation protocol? |  | [105] | c50 | c50 |
| 62 | conference establishment using request-contained lists in the session initiation protocol? |  | [106] | c51 | c52 |
| 63 | subscriptions to request-contained resource lists in the session initiation protocol? |  | [107] | c53 | c53 |
| 64 | dialstring parameter for the session initiation protocol uniform resource identifier? |  | [103] | o | c19 |
| 65 | the P-Answer-State header extension to the session initiation protocol for the open mobile alliance push to talk over cellular? |  | [111] | o | c60 |
| 66 | the SIP P-Early-Media private header extension for authorization of early media? |  | [109] 8 | o | c58 |
| 67 | number portability parameters for the 'tel' URI? |  | [112] | o | c54 |
| 67A | assert or process carrier indication? |  | [112] | o | c55 |
| 67B | local number portability? |  | [112] | o | c57 |
| 69 | extending the session initiation protocol Reason header for preemption events |  | [115] | c69 | c69 |
| 70 | communications resource priority for the session initiation protocol? |  | [116] | o | c70 |
| 70A | inclusion of MESSAGE, SUBSCRIBE, NOTIFY in communications resource priority for the session initiation protocol? |  | [116] 4.2 | c72 | c72 |
| 70B | inclusion of CANCEL, BYE, REGISTER and PUBLISH in communications resource priority for the session initiation protocol? |  | [116] 4.2 | c72 | c72 |
| 71 | addressing an amplification vulnerability in session initiation protocol forking proxies? |  | [117] | o | c87 |
| 72 | the remote application identification of applying signalling compression to SIP |  | [79] 9.1 | o | c8 |
| 73 | a session initiation protocol media feature tag for MIME application subtypes? |  | [120] | o | c59 |
| 74 | SIP extension for the identification of services? |  | [121] | o | c61 |
| 75 | a framework for consent-based communications in SIP? |  | [125] | c76 | c76 |
| 75A | a relay within the framework for consent-based communications in SIP? |  | [125] | c77 | c78 |
| 75B | a recipient within the framework for consent-based communications in SIP? |  | [125] | c80 | c79 |
| 76 | a mechanism for transporting user-to user-call control information in SIP? |  | [126] | o | c81 |
| 76A | interworking ISDN call control user information with SIP? |  | [126A] | c109 | c109 |
| 77 | The SIP P-Private-Network-Indication private-header (P-Header)? |  | [134] | o | o |
| 78 | the SIP P-Served-User private header for the 3GPP IM CN subsystem? |  | [133] 6 | o | c93 |
| 79 | the SIP P-Served-User header extension for Originating CDIV session case? |  | [239] 4 | c126 | c127 |
| 80 | marking SIP messages to be logged? |  | [140] | o | c85 |
| 81 | the 199 (Early Dialog Terminated) response code) |  | [142] | o | c86 |
| 82 | message body handling in SIP? |  | [150] | m | m |
| 83 | indication of support for keep-alive |  | [143] | o | c88 |
| 84 | SIP Interface to VoiceXML Media Services? |  | [145] | o | c89 |
| 85 | common presence and instant messaging (CPIM): message format? |  | [151] | o | c91 |
| 86 | instant message disposition notification? |  | [157] | o | c91 |
| 87 | requesting answering modes for SIP? |  | [158] | o | c60 |
| 89 | the early session disposition type for SIP? |  | [74B] | o | o |
| 91 | The Session-ID header? |  | [162] | o | c102 |
| 92 | correct transaction handling for 2xx responses to Session Initiation Protocol INVITE requests? |  | [163] | c18 | c18 |
| 93 | addressing Record-Route issues in the Session Initiation Protocol (SIP)? |  | [164] | n/a | n/a |
| 94 | essential correction for IPv6 ABNF and URI comparison in RFC3261? |  | [165] | m | m |
| 95 | suppression of session initiation protocol REFER method implicit subscription? |  | [173] | o | c99 |
| 96 | Alert-Info URNs for the Session Initiation Protocol? |  | [175] | o | o |
| 97 | multiple registrations? |  | Subclause 3.1 | n/a | c103 |
| 98 | the SIP P-Refused-URI-List private-header? |  | [183] | o | c104 |
| 99 | request authorization through dialog Identification in the session initiation protocol? |  | [184] | o | c105 |
| 100 | indication of features supported by proxy? |  | [190] | o | c106 |
| 101 | registration of bulk number contacts? |  | [191] | o | c107 |
| 102 | media control channel framework? |  | [146] | o | c108 |
| 103 | S-CSCF restoration procedures? |  | Subclause 4.14 | n/a | c110 |
| 104 | SIP overload control? |  | [198] | o | c112 |
| 104A | feedback control? |  | [199] | c113 | c113 |
| 104B | distribution of load filters? |  | [201] | c113 | c114 |
| 105 | handling of a 380 (Alternative service) response? |  | Subclauses 5.1.2A.1.1, 5.1.3.1, 5.1.6.8, and 5.2.10 | n/a | c111 |
| 106 | indication of adjacent network in the Via "received-realm" header field parameter? |  | [208] | o | c115 |
| 107 | PSAP callback indicator? |  | [209] | o | c116 |
| 108 | SIP URI parameter to indicate traffic leg? |  | [225] | o | c117 |
| 109 | PCRF based P-CSCF restoration? |  | Subclause 4.14.2 | n/a | c119 |
| 110 | HSS based P-CSCF restoration? |  | Subclause 4.14.2 | n/a | c120 |
| 111 | the Relayed-Charge header field extension? |  | Subclause 7.2.12 | n/a | c123 |
| 112 | resource sharing? |  | Subclause 4.15 | n/a | c124 |
| 113 | the Cellular-Network-Info header extension? |  | Subclause 7.2.15 | n/a | c125 |
| 114 | the Priority-Share header field extension? |  | Subclause 7.2.16 | n/a | c128 |
| 115 | the Response-Source header field extension? |  | Subclause 7.2.17 | n/a | o |
| 116 | authenticated identity management in the Session Initiation Protocol? |  | [252] | o | c129 |
| 117 | a SIP response code for unwanted calls extension? |  | [254] | o | o |
| 118 | the 3GPP PS data off extension |  | Subclause 4.17 | n/a | c130 |
| 119 | Content-ID header field in Session Initiation Protocol (SIP)? |  | [256] | o | o |
| 120 | Next-Generation Pan-European eCall emergency service? |  | [244] | o | c62 |
| 121 | the Attestation-Info header field extension? | Subclause 7.2.18 |  | n/a | c132 |
| 122 | the Origination-Id header field extension? | Subclause 7.2.19 |  | n/a | c132 |
| c2: IF A.4/20 THEN o.1 ELSE n/a - - SIP specific event notification extension. c3: IF A.3/1 OR A.3/4 OR A.3A/81 THEN m ELSE n/a - - UE or S-CSCF functional entity or MSC Server enhanced for ICS. c4: IF A.3/4 THEN m ELSE IF A.3/7 THEN o ELSE n/a - - S-CSCF or AS functional entity. c5: IF A.4/16 THEN m ELSE o - - integration of resource management and SIP extension. c6: IF A.3/4 OR A.3/1 OR A.3A/81 THEN m ELSE n/a. - - S-CSCF or UE or MSC Server enhanced for ICS. c7: IF A.3/1 OR A.3/4 OR A.3/7A OR A.3/7B OR A.3/7D OR A.3/9B OR A.3/13B OR A.3A/83 OR A.3A/89 THEN m ELSE n/a - - UA or S-CSCF or AS acting as terminating UA or AS acting as originating UA or AS performing 3rd party call control or IBCF (IMS-ALG), ISC gateway function (IMS-ALG), SCC application server, ATCF (UA). c8: IF A.3/1 THEN (IF (A.3B/1 OR A.3B/2 OR A.3B/3 OR A.3B/4 OR A.3B/5 OR A.3B/6 OR A.3B/7 OR A.3B/8 OR A.3B/11 OR A.3B/12 OR A.3B/13 OR A.3B/14 OR A.3B/15) THEN m ELSE o) ELSE n/a - - UE behaviour (based on P-Access-Network-Info usage). c9: IF A.4/26 THEN o.2 ELSE n/a - - a privacy mechanism for the Session Initiation Protocol (SIP). c10: IF A.4/26B THEN o.3 ELSE n/a - - application of privacy based on the received Privacy header. c11: IF A.3/1 OR A.3/6 OR A.3A/81 OR A.3A/81A OR A.3A/81B THEN o ELSE IF A.3/9B OR A.3/13B THEN m ELSE n/a - - UE or MGCF, IBCF (IMS-ALG), ISC gateway function (IMS-ALG), MSC Server enhanced for ICS, MSC server enhanced for SRVCC using SIP interface, MSC server enhanced for DRVCC using SIP interface. c12: IF A.3/7D OR A3A/84 OR A.3A/89 THEN m ELSE n/a - - AS performing 3rd-party call control, EATF, ATCF (UA). c13: IF A.3/1 OR A.3/2 OR A.3/4 OR A.3/9B OR A.3/11 OR A.3/12 OR A.3/13B OR A.3A/81 THEN m ELSE o - - UE or S-CSCF or IBCF (IMS-ALG) or E-CSCF or LRF or ISC gateway function (IMS-ALG) or MSC Server enhanced for ICS. c14: IF A.3/1 AND A4/2B AND (A.3B/1 OR A.3B/2 OR A.3B/3) THEN m ELSE IF A.3/2 THEN o ELSE n/a – UE and initiating sessions and GPRS IP-CAN or P-CSCF. c15: IF A.4/20 AND (A.3/4 OR A.3/9B OR A.3/11 OR A.3/13B) THEN m ELSE o – SIP specific event notification extensions and S-CSCF or IBCF (IMS-ALG) or E-CSCF or ISC gateway function (IMS-ALG). c16: IF A.4/20 AND (A.3/1 OR A.3/2 OR A.3/9B OR A.3/12 OR A.3/13B OR A.3A/81) THEN m ELSE o - - SIP specific event notification extension and UE or P-CSCF or IBCF (IMS-ALG) or MSC Server enhanced for ICS or LRF or ISC gateway function (IMS-ALG). c17: IF A.3/1 OR A.3/4 OR A.3A/81 THEN m ELSE n/a - - UE or S-CSCF or MSC Server enhanced for ICS. c18: IF A.4/2B THEN m ELSE n/a - - initiating sessions. c19: IF A.4/2B THEN o ELSE n/a - - initiating sessions. c20: IF A.3/1 AND (A.3D/1 OR A.3D/4) THEN m ELSE n/a - - UE and (IMS AKA plus IPsec ESP or SIP digest with TLS). c21: IF A.4/30 THEN o.4 ELSE n/a - - private header extensions to the session initiation protocol for the 3rd-Generation Partnership Project (3GPP). c22: IF A.4/30 AND (A.3/1 OR A.3/4 OR A.3A/81) THEN m ELSE n/a - - private header extensions to the session initiation protocol for the 3rd-Generation Partnership Project (3GPP) and S-CSCF or UE or MSC Server enhanced for ICS. c23: IF A.4/30 AND (A.3/1 OR A.3A/81) THEN o ELSE n/a - - private header extensions to the session initiation protocol for the 3rd-Generation Partnership Project (3GPP) and UE or MSC Server enhanced for ICS. c24: IF A.4/30 AND (A.3/4 OR A.3A/81 OR A.3A/81A) THEN m ELSE n/a - - private header extensions to the session initiation protocol for the 3rd-Generation Partnership Project (3GPP) and S-CSCF or MSC Server enhanced for ICS or MSC server enhanced for SRVCC using SIP interface. c25: IF A.4/30 AND (A.3A/81 OR A.3/4 OR A.3/6 OR A.3/7A OR A.3/7D OR A.3/9B OR A.3/13B OR A3A/84 OR A.3A/81A OR A.3A/81B) THEN m ELSE IF A.4/30 AND A.3/1 AND (A.3B/1OR A.3B/2 OR A.3B/3 OR A.3B/4 OR A.3B/5 OR A.3B/6 OR A.3A/7 OR A.3A/8 OR A.3B/11OR A.3B/12 OR A.3B/13 OR A.3B/14 OR A.3A/15 OR A.3B/41) THEN m ELSE IF A4/30 AND A.3/1 AND (A.3B/21 OR A.3B/22 OR A.3B/23 OR A.3B/24 OR A.3B/25 OR A.3B/26 OR A.3A/27 OR A.3A/28 OR A.3B/29 OR A.3B/30) THEN o ELSE n/a - - private header extensions to the session initiation protocol for the 3rd-Generation Partnership Project (3GPP), MSC Server enhanced for ICS, S-CSCF, MGCF or AS acting as terminating UA or AS acting as third-party call controller or IBCF (IMS-ALG), ISC gateway function (IMS-ALG), UE, EATF, P-Access-Network-Info values or MSC server enhanced for SRVCC using SIP interface, MSC server enhanced for DRVCC using SIP interface. c26: IF A.4/30 AND (A.3A/81 OR (A.3/4 AND A.4/2) OR A.3/6 OR A.3/7A OR A.3/7B or A.3/7D OR A.3/9B OR A.3/13B OR A3A/84 OR A.3A/89 OR A.3A/81A OR A.3A/81B) THEN m ELSE n/a - - private header extensions to the session initiation protocol for the 3rd-Generation Partnership Project (3GPP) MSC Server enhanced for ICS, S-CSCF, registrar, MGCF, AS acting as a terminating UA, or AS acting as an originating UA, or AS acting as third-party call controller, IBCF (IMS-ALG), ISC gateway function (IMS-ALG), EATF, ATCF (UA), MSC server enhanced for SRVCC using SIP interface, MSC server enhanced for DRVCC using SIP interface. c27: IF A.3/7D OR A.3/9D THEN o ELSE x - - AS performing 3rd party call control, IBCF (Privacy). c29: IF A.4/40A OR A.4/40B OR A.4/40C OR A.4/40D OR A.4/40E OR A.4/40F THEN m ELSE n/a - - support of any directives within caller preferences for the session initiation protocol. c30: IF A.3A/1 OR A.3A/2 THEN m ELSE IF A.3/1 OR A.3/11A OR A.3/2A THEN o ELSE n/a - - presence server, presence user agent, UE, AS, E-CSCF acting as UA, P-CSCF (IMS-ALG). |  |  |  |  |  |
| c33: IF A.3/9B OR A.3/12 OR A.3/13B OR A.3A/81 OR A.3A/11 OR A.3A/12 OR A.4/44 OR A.3A/81A OR A.3A/81B THEN m ELSE o - - IBCF (IMS-ALG) or LRF or ISC gateway function (IMS-ALG) or MSC Server enhanced for ICS or conference focus or conference participant or the Session Inititation Protocol (SIP) "Replaces" header, MSC server enhanced for SRVCC using SIP interface, MSC server enhanced for DRVCC using SIP interface. c34: IF A.4/44 OR A.4/45 OR A.3/9B OR A.3/13 THEN m ELSE n/a - - the Session Inititation Protocol (SIP) "Replaces" header or the Session Inititation Protocol (SIP) "Join" header or IBCF (IMS-ALG) or ISC gateway function (IMS-ALG). c35: IF A.3/4 OR A.3/9B OR A.3/13B OR A.3A/82 OR A.3A/83 OR A.3A/21 OR A.3A/22 OR A3A/84 THEN m ELSE IF (A.3/1 OR A.3/6 OR A.3/7 OR A.3/8 OR A.3A/81 OR A.3A/81A OR A.3A/81B) THEN o ELSE n/a - - S-CSCF or IBCF (IMS-ALG) or ISC gateway function (IMS-ALG) functional entities or ICS user agent or SCC application server or CSI user agent or CSI application server, UE or MGCF or AS or MRFC functional entity or MSC Server enhanced for ICS or EATF or MSC server enhanced for SRVCC using SIP interface, MSC server enhanced for DRVCC using SIP interface. c37 IF A.4/47 THEN o.3 ELSE n/a - - an extension to the session initiation protocol for request history information. c38: IF A.4/2B AND (A.3A/11 OR A.3A/12 OR A.3/7D) THEN m ELSE IF A.4/2B THEN o ELSE n/a - - initiating sessions, conference focus, conference participant, AS performing 3rd party call control. c39: IF A.3/1 THEN m ELSE IF A.3/7B OR A.3/7D OR A.3/9 THEN o ELSE n/a - - UE, AS acting as an originating UA, or AS acting as third-party call controller, IBCF. c40 IF A.3/4 OR (A.3/1 AND NOT A.3C/1) OR A.3A/81 OR A.4/22 THEN m ELSE IF (A.3/7A OR A.3/7B OR A.3/7D) THEN o ELSE n/a - - S-CSCF, UE, UE performing the functions of an external attached network, MSC Server enhanced for ICS, notifier of event information, AS, AS acting as terminating UA, or redirect server, AS acting as originating UA, AS performing 3rd party call control. c42: IF A.3/1 THEN n/a ELSE o - - UE. c43: IF A.4/2B THEN o ELSE n/a - - initiating sessions. c44: IF A.4/2C THEN m ELSE o - - initiating a session which require local and/or remote resource reservation. c45: IF A.4/97 THEN m ELSE n/a - - multiple registrations. c46 IF A.3/1 OR A.3/4 THEN o ELSE n/a - - UE, S-CSCF. c47: IF A.4/27 THEN o ELSE n/a - - a messaging mechanism for the Session Initiation Protocol (SIP). c48: IF A.3A/32 AND A.4/27 THEN m ELSE IF A.4/27 THEN o ELSE n/a - - messaging list server, a messaging mechanism for the Session Initiation Protocol (SIP). c49: IF A.3/1 OR A.3/9B OR A.3/13B OR A.3A/81 OR A.3/11 OR A.3/12 OR A3A/84 THEN m ELSE o - - UE, IBCF (IMS-ALG), ISC gateway function (IMS-ALG), MSC Server enhanced for ICS, E-CSCF, LRF, EATF. c50: IF A.3A/81 OR A.3A/81A OR A.3A/81B THEN n/a ELSE IF A.4/15 THEN o ELSE n/a - - MSC Server enhanced for ICS, MSC server enhanced for SRVCC using SIP interface, MSC server enhanced for DRVCC using SIP interface, the REFER method. c51: IF A.4/2B THEN o ELSE n/a - - initiating a session. c52: IF A.3A/11 AND A.4/2B THEN m ELSE IF A.4/2B THEN o ELSE n/a - - conference focus, initiating a session. c53: IF A.3A/81 THEN n/a ELSE IF A.4/20 THEN o ELSE n/a - - MSC Server enhanced for ICS, SIP specific event notification. c54: IF A.3/1 OR A.3/6 OR A.3/7A OR A.3/7D OR A.3/9 THEN o, ELSE n/a - - UE, MGCF, AS acting as originating UA, AS performing 3rd party call control, IBCF. c55: IF A.4/67 THEN m ELSE n/a - - number portability parameters for the 'tel' URI. c57: IF A.4/67 THEN m ELSE n/a - - number portability parameters for the 'tel' URI. c58: IF A.3/9B OR A.3/13B OR A.3/6 OR A.3A/81 OR A.3A/81A OR A.3A/81B THEN m ELSE o - - IBCF (IMS-ALG), ISC gateway function (IMS-ALG), MGCF, MSC Server enhanced for ICS, MSC server enhanced for SRVCC using SIP interface, MSC server enhanced for DRVCC using SIP interface. c59: IF A.3/4 THEN m ELSE IF (A.3/1 OR A.3/6 OR A.3/7A OR A.3/7B OR A.3/7D OR A.3/8) THEN o ELSE n/a - - S-CSCF, UE, MGCF, AS, AS acting as terminating UA, or redirect server, AS acting as originating UA, AS performing 3rd party call control, or MRFC. c60: IF A.3/9B OR A.3/13B THEN m ELSE IF A.3/1 OR A.3/7A OR A.3/7B OR A.3/7D THEN o ELSE n/a - - IBCF (IMS-ALG), ISC gateway function (IMS-ALG), UE, AS acting as terminating UA, AS acting as originating UA, AS performing 3rd party call control. c61: IF (A.3/1 OR A.3A/81 OR A.3/6 OR A.3/7A OR A.3/7B OR A.3/7D OR A.3/8 OR A.3/9B OR A.3/13 OR A3A/84 OR A.3A/81A OR A.3A/81B) THEN o ELSE n/a - - UE, MSC Server enhanced for ICS, MGCF, AS, AS acting as terminating UA, or redirect server, AS acting as originating UA, AS performing 3rd party call control, or MRFC or IBCF (IMS-ALG), ISC gateway function (IMS-ALG), EATF, MSC server enhanced for SRVCC using SIP interface, MSC server enhanced for DRVCC using SIP interface. c62: IF A.3/1 THEN o ELSE n/a - - UE. c68: IF A.3/2A OR A.3/9 OR A.4/69 OR A.3A/83 THEN m ELSE o - - P-CSCF (IMS-ALG), IBCF, extending the session initiation protocol Reason header for preemption events and Q.850 causes, SCC application server. c69: IF A.4/70 THEN o ELSE n/a - - communications resource priority for the session initiation protocol. c70: IF A.3/9B OR A.3/13B OR A.3A/102 OR A.3A/103 THEN m ELSE IF A.3/1 OR A.3/6 OR A.3/7 OR A.3/7A OR A.3/7B OR A.3/7D OR A.3A/81 OR A.3A/81A OR A.3A/81B THEN o ELSE n/a - - IBCF (IMS-ALG), ISC gateway function (IMS-ALG), MCPTT client, MCPTT server, UE, MGCF, AS, AS acting as terminating UA, or redirect server, AS acting as originating UA, AS performing 3rd party call control, MSC Server enhanced for ICS, MSC server enhanced for SRVCC using SIP interface, MSC server enhanced for DRVCC using SIP interface. c72: IF A.4/70 THEN o ELSE n/a - - communications resource priority for the session initiation protocol |  |  |  |  |  |
| c74: IF A.3/4 OR A.3/1 THEN o ELSE n/a. - - S-CSCF or UE. c75: IF A.3/1 THEN o ELSE n/a. - - UE. c76: IF A.4/75A OR A.4/75B THEN m ELSE n/a - - a relay within the framework for consent-based communications in SIP, a recipient within the framework for consent-based communications in SIP. c77: IF A.4/59 OR A.4/61 OR A.4/62 OR A.4/63 THEN m ELSE o - - multiple-recipient MESSAGE requests in the session initiation protocol, referring to multiple resources in the session initiation protocol, conference establishment using request-contained lists in the session initiation protocol, subscriptions to request-contained resource lists in the session initiation protocol. c78: IF (A.4/59 OR A.4/61 OR A.4/62 OR A.4/63) AND (A.3A/11 OR A.3A/31) THEN m ELSE o - - multiple-recipient MESSAGE requests in the session initiation protocol, referring to multiple resources in the session initiation protocol, conference establishment using request-contained lists in the session initiation protocol, subscriptions to request-contained resource lists in the session initiation protocol, conference focus, messaging application server. c79: IF A.3/9B OR A.3/13B OR (A.3/1 AND (A.4/2B OR A.4/15 OR A.4/20 OR A.4/27)) THEN m ELSE IF A.3/6 OR A.3/7A OR A.3/7D THEN o ELSE n/a - - IBCF (IMS-ALG), ISC gateway function (IMS-ALG), UE, initiating a session, the REFER method, SIP specific event notification, a messaging mechanism for the Session Initiation Protocol (SIP), AS acting as terminating UA, or redirect server, AS performing 3rd party call control. c80: IF A.4/2B OR A.4/15 OR A.4/20 OR A.4/27 THEN m ELSE n/a - - initiating a session, the REFER method, SIP specific event notification, a messaging mechanism for the Session Initiation Protocol (SIP). c81: IF A.3/1 OR A.3/6 OR A.3/7A OR A.3/7B OR A.3/7D THEN o ELSE IF A.3/9B OR A.3/13B THEN m ELSE n/a - - UE, MGCF, AS acting as terminating UA, or redirect server, AS acting as originating UA, AS performing 3rd party call control, IBCF (IMS-ALG), ISC gateway function (IMS-ALG). c82: IF A.3/6 OR A.3A/81 OR A.3A/81A OR A.3A/81B THEN m ELSE n/a - - MGCF, MSC server enhanced for ICS, MSC server enhanced for SRVCC using SIP interface, MSC server enhanced for DRVCC using SIP interface. c85: IF A.3/1 OR A.3/6 OR A.3A/81 OR A.3A/81A OR A.3A/81B OR A.3/2 OR A.3/7B THEN m ELSE n/a - - UE, MGCF, MSC Server enhanced for ICS, MSC Server enhanced for SRVCC using SIP interface, MSC Server enhanced for DRVCC using SIP interface, P-CSCF, AS acting as originating UA. c86: IF A.4/3 OR A.4/4 THEN m ELSE n/a - - client behaviour for INVITE requests, server behaviour for INVITE requests. c87: IF A.3/9B OR A.3/9C OR A.3/13B OR A.3/13C THEN m ELSE o - - IBCF (IMS-ALG), IBCF (Screening of SIP signalling), ISC gateway function (IMS-ALG), ISC gateway function (Screening of SIP signalling). c88: IF A.3/1 OR A.3/2 THEN m ELSE o - - UE, P-CSCF. c89: IF A.3/7A OR A.3/8 THEN o ELSE n/a - - AS performing 3rd party call control, MRFC. c90: IF A.4/13 OR A.3A/53 OR A.3A/54 OR A.3A/91 OR A.3A/85 OR A.3A/86 THEN m ELSE o - - SIP INFO method and package framework, advice of charge application server, advice of charge UA client, malicious communication identification application server, in-dialog overlap signalling application server, in-dialog overlap signalling UA client. c91: IF A.3A/61 OR A.3A/62 OR A.3A/63 OR A.3A/71 THEN m ELSE o - - SM-over-IP sender, SM-over-IP receiver, IP-SM-GW, IP-SM-GW. c93: IF A.3/7B OR A.3/7D OR A3A/84 THEN o ELSE n/a - - AS acting as originating UA, AS performing 3rd party call control, EATF. c94: IF A.3/4 OR A.3/7A OR A.3/7D THEN o ELSE n/a - - S-CSCF and AS acting as terminating UA or redirect server or AS performing 3rd party call control. c96: IF A.4/30 THEN o ELSE n/a - - extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks. c97: IF (A.3/9B OR A.3/9C OR A.3/13B OR A.3/13C) AND A.4/30 THEN m ELSE IF (A.3/7D OR A.3/11 OR A.3C/1) AND A.4/30 THEN o ELSE n/a - - IBCF (IMS-ALG), IBCF (Screening of SIP signalling), ISC gateway function (IMS-ALG), ISC gateway function (Screening of SIP signalling), AS performing 3rd party call control, E-CSCF, UE performing the functions of an external attached network and extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks. c98: IF A.3/7D OR A.3/9B OR A.3/9C OR A.3/13B OR A.3/13C OR A.3C/1 OR A3A/84 OR A.3A/89 THEN m ELSE n/a - - AS performing 3rd party call control, IBCF (IMS-ALG), IBCF (Screening of SIP signalling), ISC gateway function (IMS-ALG), ISC gateway function (Screening of SIP signalling), UE performing the functions of an external attached network, EATF, ATCF (UA). c99: IF A.4/15 AND (A.3/9B OR A.3/9C OR A.13/B OR A.13/C) THEN m ELSE IF A.4/15 THEN o ELSE n/a - - the REFER method, IBCF (IMS-ALG), IBCF (Screening of SIP signalling), ISC gateway function (IMS-ALG), ISC gateway function (Screening of SIP signalling). c100: IF A.3/6 OR A.3A/57 OR A.3A/58 OR A.3A/59 OR A.3A/60 OR A.3A/81 OR A.3A/81A OR A.3A/81B THEN m ELSE o - - MGCF, customized alerting tones application server, customized alerting tones UA client, customized ringing signal application server, customized ringing signal UA client, MSC server enhanced for ICS, MSC server enhanced for SRVCC using SIP interface, MSC server enhanced for DRVCC using SIP interface. c101: IF A.3D/30 OR A.3D/20A OR A.3D/20B OR A.3D/20C THEN m ELSE n/a - - end-to-access-edge media security using SDES, end-to-access-edge media security for MSRP using TLS and certificate fingerprints, end-to-access-edge media security for BFCP using TLS and certificate fingerprints, end-to-access-edge media security for UDPTL using DTLS and certificate fingerprints. c102: IF A.3A/11 OR A.3A/12 OR A.3/9 THEN m ELSE n/a - - conference focus, conference participant, IBCF. c103: IF A.3/1 THEN o ELSE IF A.3/2 OR A.3/4 THEN m ELSE n/a - - UE, P-CSCF, S-CSCF. c104: IF A.3/9B OR A.3/13B THEN m ELSE IF A.3/7A OR A.3/7B OR A.3/7D THEN o ELSE n/a - - IBCF (IMS-ALG), ISC gateway function (IMS-ALG), AS acting as terminating UA, AS acting as originating UA, AS performing 3rd party call control. |  |  |  |  |  |
| c105: IF A.3/9B OR A.3/13B OR A.3A/82 OR A.3A/83 OR A.3A/87 OR A.3A/89 THEN m ELSE o - - IBCF (IMS-ALG), ISC gateway function (IMS-ALG), ICS user agent, SCC application server, Session continuity controller UE, ATCF (UA). c106: IF A.3A/50A OR A.3A/83 OR A.3A/89 THEN m ELSE o - - Multimedia telephony application server, SCC application server, ATCF (UA). c107: IF A.3C/1 OR A.4/2 THEN o ELSE n/a - - UE performing the functions of an external attached network, registrar. c108: IF A.3/7 OR A.3/8 OR A.3/8A THEN o ELSE n/a - - AS, MRFC, MRB. c109: IF A.4/76 THEN o ELSE n/a - - a mechanism for transporting user to user call control information in SIP. c110: IF A.3/1 THEN m ELSE IF A.3/2 OR A.3/3 OR A.3/4 THEN o ELSE n/a - - UE, P-CSCF, I-CSCF, S-CSCF. c111: IF A.3/1 OR A.3/2 THEN m ELSE n/a - - UE, P-CSCF. c112: IF NOT (A.3/1 AND NOT A.3C/1) THEN o ELSE n/a - - not UE, UE performing the functions of an external attached network. c113: IF A.4/104 THEN o.7 ELSE n/a - - SIP overload control. c114: IF A.4/104 THEN IF A.3/4 OR A.3/7 OR A.3/10 THEN o.7 ELSE n/a - - SIP overload control, S-CSCF, AS, additional routeing functionality. c115: IF A.3/6 OR A.3/9 OR A.3/7 THEN o ELSE n/a - - MGCF, IBCF, AS c116: IF A.3/2A OR A.3/6 OR A.3/7 OR A.3/9 THEN o ELSE IF A.3/1 THEN x ELSE n/a - - P-CSCF (IMS-ALG), MGCF, AS, IBCF, UE. c117 IF A.3/2 OR A.3/4 OR OR A.3/9 OR A.3A/81 OR A.3A/83 OR A.3A/89 OR A.3A/81A THEN o ELSE n/a - - P-CSCF, S-CSCF, IBCF, MSC server enhanced for ICS, SCC application server, ATCF (UA), MSC server enhanced for SRVCC using SIP interface. c118: IF A.4/49 THEN o ELSE n/a - - session initiation protocol URIs for applications such as voicemail and interactive voice response (NOTE 3). c119: IF A.3/2A OR A.3/9 THEN o ELSE n/a - - P-CSCF (IMS-ALG), IBCF. c120: IF A.3/2A OR A.3/9 THEN o ELSE n/a - - P-CSCF (IMS-ALG), IBCF. c121: IF A.4/15 THEN m ELSE n/a - - the REFER method. c122: IF A.4/22 THEN m ELSE n/a - - act as a notifier. c123: IF A.4/111 AND (A.3/7A OR A.3/7B OR A.3/9A OR A.3/9B OR A.3/13A OR A.3/13B) THEN m ELSE IF A.3/4 OR A.3/7 OR A.3A/102 THEN o ELSE n/a.-.-.the Relayed-Charge header field extension, AS acting as terminating UA, or redirect server, AS acting as originating UA, IBCF (THIG), IBCF (IMS-ALG), ISC gateway function (THIG), ISC gateway function (THIG), S-CSCF, AS, transit function. c124: IF A.3/2A OR A.3/9B OR A.3/7 THEN o ELSE n/a - - P-CSCF (IMS-ALG), I-BCF (IMS-ALG), AS. c125: IF (A.3/4 OR A.3/6 OR A.3/7A OR A.3/7D OR A.3/9B OR A.3/13B OR A.3A/84 OR A.3A/89 OR A.3/2A OR A.3/8 OR A.3/11A) THEN m ELSE IF A.3/1 AND (A.3B/11 OR A.3B/12 OR A.3B/13 OR A.3B/14 OR A.3B/15) AND (A.3B/1 OR A.3B/2 OR A.3B/3 OR A.3B/4 OR A.3B/5 OR A.3B/6 OR A.3B/7 OR A.3B/8 OR A.3B/9) THEN m ELSE n/a. - - S-CSCF, MGCF, AS acting as terminating UA, AS acting as third-party call controller, IBCF (IMS-ALG), ISC gateway function (IMS-ALG), EATF, ATCF acting as UA, P-CSCF (IMS-ALG), MRFC, E-CSCF acting as UA, UE. c126: IF A.4/78 THEN o ELSE n/a - - the SIP P-Served-User private header for the 3GPP IM CN subsystem. c127: IF A.4/78 THEN m ELSE n/a - - the SIP P-Served-User private header for the 3GPP IM CN subsystem. c128: IF A.3/2A OR A.3/9B OR A.3/7 OR A.3A/103 THEN o ELSE n/a - - P-CSCF (IMS-ALG), IBCF (IMS-ALG), AS, MCPTT server. c129 IF A.3/6 OR A.3/7 OR A.3/9 OR A.3A/81 OR A.3A/81A OR A.3A/81B THEN o ELSE n/a - - MGCF, AS, IBCF, MSC Server enhanced for ICS, MSC server enhanced for SRVCC using SIP interface, MSC server enhanced for DRVCC using SIP interface. c130: IF A.3/1 OR A.3/7 THEN o ELSE n/a - - UE, AS, c131: IF A.3/6 OR A.3A/81 OR A.3A/81A OR A.3A/81B THEN o ELSE n/a - - MGCF, MSC server enhanced for ICS, MSC server enhanced for SRVCC using SIP interface, MSC server enhanced for DRVCC using SIP interface. c132: IF A.3/6 OR A.3/7 OR A.3/9 THEN o ELSE n/a - - MGCF, AS, IBCF. c133 IF A.4/60 THEN o ELSE n/a - - the Geolocation header field o.1: At least one of these capabilities is supported. o.2: At least one of these capabilities is supported. o.3: At least one of these capabilities is supported. o.4: At least one of these capabilities is supported. o.5: At least one of these capabilities is supported. o.6: It is mandatory to support at least one of these items. o.7: At least one of these capabilities is supported. |  |  |  |  |  |
| NOTE 1: An AS acting as a proxy may be outside the trust domain, and therefore not able to support the capability for that reason; in this case it is perfectly reasonable for the header to be passed on transparently, as specified in the PDU parts of the profile. NOTE 2: If a UE is unable to become engaged in a service that potentially requires the ability to identify and interact with a specific UE even when multiple UEs share the same single Public User Identity then the UE support can be "o" instead of "m". Examples include telemetry applications, where point-to-point communication is desired between two users. NOTE 3: AS performing a service number translation (eg. Freephone) |  |  |  |  |  |

Prerequisite A.4/20 - - SIP specific event notification

Table A.4A: Supported event packages

| Item | Does the implementation support | Subscriber |  |  | Notifier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | reg event package? | [43] | c1 | c3 | [43] | c2 | c4 |
| 1A | reg event package extension for GRUUs? | [94] | c1 | c25 | [94] | c2 | c4 |
| 2 | refer package? | [36] 3 | c13 | c13 | [36] 3 | c13 | c13 |
| 3 | presence package? | [74] 6 | c1 | c5 | [74] 6 | c2 | c6 |
| 4 | eventlist with underlying presence package? | [75], [74] 6 | c1 | c7 | [75], [74] 6 | c2 | c8 |
| 5 | presence.winfo template-package? | [72] 4 | c1 | c9 | [72] 4 | c2 | c10 |
| 6 | xcap-diff package? | [77] 4 | c1 | c11 | [77] 4 | c2 | c12 |
| 7 | conference package? | [78] 3 | c1 | c21 | [78] 3 | c1 | c22 |
| 8 | message-summary package? | [65] | c1 | c23 | [65] 3 | c2 | c24 |
| 9 | poc-settings package? | [110] | c1 | c26 | [110] | c2 | c27 |
| 11 | dialog event package? | [171] | c1 | c14 | [171] | c2 | c15 |
| 12 | load-control package? | [201] | c29 | c30 | [201] | c29 | c31 |
| c1: IF A.4/23 THEN o ELSE n/a - - acting as the subscriber to event information. c2: IF A.4/22 THEN o ELSE n/a - - acting as the notifier of event information. c3: IF A.3/1 OR A.3A/81 OR A.3/2 THEN m ELSE IF A.3/7 THEN o ELSE n/a - - UE, MSC Server enhanced for ICS, P-CSCF, AS. c4: IF A.3/4 THEN m ELSE IF A.3C/1 THEN o ELSE n/a - - S-CSCF, UE performing the functions of an external attached network. c5: IF A.3A/3 OR A.3A/4 THEN m ELSE IF A.4/23 OR A.3/12 THEN o ELSE n/a - - resource list server or watcher, acting as the subscriber to event information, LRF. c6: IF A.3A/1 THEN m ELSE IF A.4/22 OR A.3/11A THEN o ELSE n/a - - presence server, acting as the notifier of event information, E-CSCF acting as UA. c7: IF A.3A/4 THEN m ELSE IF A.4/23 THEN o ELSE n/a - - watcher, acting as the subscriber to event information. c8: IF A.3A/3 THEN m ELSE IF A.4/22 THEN o ELSE n/a - - resource list server, acting as the notifier of event information. c9: IF A.3A/2 THEN m ELSE IF A.4/23 THEN o ELSE n/a - - presence user agent, acting as the subscriber to event information. c10: IF A.3A/1 THEN m ELSE IF A.4/22 THEN o ELSE n/a - - presence server, acting as the notifier of event information. c11: IF A.3A/2 OR A.3A/4 OR A.3A/56 THEN o ELSE IF A.4/23 THEN o ELSE n/a - - presence user agent or watcher or Ut reference point XCAP client for supplementary services, acting as the subscriber to event information. c12: IF A.3A/1 OR A.3A/3 OR A.3A/55 THEN m ELSE IF A.4/22 THEN o ELSE n/a - - presence server or resource list server or Ut reference point XCAP server for supplementary services, acting as the notifier of event information. c13: IF A.4/15 THEN m ELSE n/a - - the REFER method. c14: IF A.3/12 OR A.3A/87 THEN m ELSE IF A.3/1 OR A.3/7B OR A.3/7D THEN o ELSE n/a - - LRF, session continuity controller UE, UE, AS acting as originating UA, AS performing 3rd party call control. c15: IF A.3/11 OR A.3A/83 THEN m ELSE IF A.3/1 OR A.3/7A OR A.3/7D THEN o ELSE n/a - - E-CSCF, SCC application server, UE, AS acting as terminating UA, or redirect server, AS performing 3rd party call control. c21: IF A.3A/12 THEN m ELSE IF A.4/23 THEN o ELSE n/a - - conference participant or acting as the subscriber to event information. c22: IF A.3A/11 THEN m ELSE IF A.4/22 THEN o ELSE n/a - - conference focus or acting as the notifier of event information. c23: IF A.3A/52 THEN m ELSE (A.3/1 OR A.3/7A OR A.3/7B) AND A.4/23 THEN o ELSE n/a - - message waiting indication subscriber UA, UE, AS acting as terminating UA, or redirect server, AS acting as originating UA all as subscriber of event information. c24: IF A.3A/52 THEN m ELSE (A.3/1 OR A.3/7A OR A.3/7B) AND A.4/22 THEN o ELSE n/a - - message waiting indication notifier UA, UE, AS acting as terminating UA, or redirect server, AS acting as originating UA all as notifier of event information. c25: IF A.4A/1 THEN (IF A.3/1 AND A.4/53 THEN m ELSE o) ELSE n/a - - reg event package, UE, reg event package extension for GRUUs. c26: IF (A.3/7B OR A.3/1) AND (A.4/23 OR A.4/41) THEN o ELSE n/a - - AS acting as originating UA, UE, acting as the subscriber to event information, an event state publication extension to the session initiation protocol. c27: IF (A.4/22 OR A.4/41) AND A.3/1 THEN o ELSE n/a - - UE, acting as the notifier of event information, an event state publication extension to the session initiation protocol. c28: IF A.3/1 OR A.3A/81 OR A.3/2 OR A.3/7B THEN m ELSE n/a - - UE, MSC Server enhanced for ICS, P-CSCF, AS acting as originating UA. c29: IF A.4/104B THEN m ELSE n/a - - distribution of load filters. c30: IF A.4/104B THEN IF A.3/4 OR A.3/7 OR A.3/9 THEN m ELSE n/a - - distribution of load filters. S-CSCF, IBCF, AS. c31: IF A.4/104B THEN If A.3/7 THEN m ELSE n/a - - distribution of load filters, AS. |  |  |  |  |  |  |  |

Prerequisite A.4/13 - - SIP INFO method and package framework.

Table A.4B: Supported info packages

| Item | Does the implementation support | Sender |  |  | Receiver |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | DTMF info package? | 7.12.1 | n/a | c1 | 7.12.1 | n/a | c1 |
| 2 | g.3gpp.mid-call? | [8M] | n/a | c2 | [8M] | n/a | c3 |
| 3 | g.3gpp.ussd? | [8W] | n/a | c4 | [8W] | n/a | c4 |
| 4 | g.3gpp.current-location-discovery info package ? | subclause 7.12.2.1 | n/a | c5 | subclause 7.12.2.1 | n/a | c6 |
| 5 | EmergencyCallData.eCall.MSD Info-Package | [244] 14.9 | m | c7 | [244] 14.9 | m | c7 |
| c1: IF A.3/6 OR A.3A/57 OR A.3A/58 OR A.3A/59 OR A.3A/60 THEN m ELSE o - - MGCF, customized alerting tones application server, customized alerting tones UA client, customized ringing signal application server, customized ringing signal UA client. c2: IF A.3A/83 THEN o ELSE n/a - - SCC application server. c3: IF A.3A/81 OR A.3A/81B THEN o ELSE n/a - - MSC server enhanced for ICS, MSC server enhanced for DRVCC using SIP interface. c4: IF A.3A/92 OR A.3A/93 THEN m ELSE n/a - - USSI UE, USSI AS. c5: IF A.3/11A OR A.3/2A THEN o ELSE n/a - - E-CSCF acting as UA, P-CSCF (IMS-ALG). c6: IF (A.3/1 AND (A.3B/11 OR A.3B/12 OR A.3B/13 OR A.3B/14 OR A.3B/15)) OR A.3/2A THEN o ELSE n/a - - UE, IEEE-802.11, IEEE-802.11a, IEEE-802.11b, IEEE-802.11g, IEEE-802.11n, P-CSCF (IMS-ALG). c7: IF (A.3/1 AND A.4/120) THEN m ELSE n/a - - UE, Next-Generation Pan-European eCall emergency service. |  |  |  |  |  |  |  |

Table A.4C: Supported media control packages

| Item | Does the implementation support | Sender |  |  | Receiver |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | msc-ivr/1.0 | [147] |  | c1 | [147] |  | c2 |
| 2 | msc-mixer/1.0 | [148] |  | c1 | [148] |  | c2 |
| 3 | mrb-publish/1.0 | [192] |  | c3 | [192] |  | c4 |
| c1: IF A.3/7D THEN o ELSE n/a - - AS performing 3rd party call control. c2: IF A.3/8 THEN o ELSE n/a - - MRFC. c3: IF A.3/8 THEN o ELSE n/a - - MRFC. c4: IF A.3/8A THEN o ELSE n/a - - MRB. |  |  |  |  |  |  |  |

End of changes