3GPP TSG CT WG6 Meeting #92	C6-190056

# Montreal, Canada, 26 January - 1 February 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.102 | CR | 0840 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of length in 5GS3GPPLOCI and 5GSN3GPPLOCI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Gemalto |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-CT |  |  |  |  |  | Date: |  |  | 27/02/2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The lengths of 5G-GUTI for 3GPP access and 5G-GUTI for non-3GPP access as currently defined in EF5GS3GPPLOCI and EF5GSN3GPPLOCI respectively are incorrect according to 3GPP TS 24.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify the correct length for 5G-GUTI for 3GPP access and 5G-GUTI for non-3GPP access which is 13 bytes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS 31.102 specification is not aligned with TS 24.501 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.11.2, 4.4.11.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** Beginning of change ***

#### 4.4.11.2	EF5GS3GPPLOCI (5GS 3GPP location information)

If service n°122 is "available" in EFUST, this file shall be present.

This EF contains the following 5GS location information for 3GPP access:

-	5G-Globally Unique Temporary Identifier (5G-GUTI);

-	Last visited registered Tracking Area Identity in 5GS (TAI);

-	5GS update status.

| Identifier: '4F01' |  | Structure: transparent |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| SFI: '01' |  |  |  |  |  |  |
| File size: 20 bytes |  |  | Update activity: high |  |  |  |
| Access Conditions: READ PIN UPDATE PIN DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 to 13 | 5G-GUTI for 3GPP access |  |  | M |  | 13 bytes |
| 14 to19 | Last visited registered TAI in 5GS for 3GPP access |  |  | M |  | 6 bytes |
| 20 | 5GS update status for 3GPP access |  |  | M |  | 1 byte |

-	5G-GUTI for 3GPP access.

Contents:

5G-Globally Unique Temporary Identifier for 3GPP access.

Coding:

as the 5G-GUTI part of the 5GS mobile identity information element defined in TS 24.501 [104]. Byte 1 corresponds to "octet 2" of an 5GS mobile identity information element containing a 5G-GUTI. Byte 13 corresponds to "octet 14" of an 5GS mobile identity information element information element containing a 5G-GUTI.

Byte 1: first byte of 5G-GUTI for 3GPP access

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MSB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

-	Last visited registered TAI in 5GS for 3GPP access

Contents:

Last visited registered Tracking Area Identity in 5GS for 3GPP access.

Coding:

as the content of the tracking area identity information element defined in TS 24.501 [104]. Byte 14 corresponds to "octet 2" of a tracking area identity information element. Byte 19 corresponds to "octet 7" of a tracking area identity information element.

Byte 14: first byte of last visited registered TAI for 3GPP access

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MSB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

-	5GS update status for 3GPP access.

Contents:

status of 5GS update for 3GPP access according to TS 24.501 [104].

Coding:

byte 20:

Bits:	b3	b2	b1.

0	0	0	:	5U1 UPDATED.

0	0	1	:	5U2 NOT UPDATED.

0	1	0	:	5U3 ROAMING NOT ALLOWED.

0	1	1	:	reserved.

1	0	0	:	reserved.

1	0	1	:	reserved.

1	1	0	:	reserved.

1	1	1	:	reserved.

Bits b4 to b8 are RFU (see TS 31.101 [11]).

Unused bytes shall be set to 'FF'.

*** Next change ***

#### 4.4.11.3	EF5GSN3GPPLOCI (5GS non-3GPP location information)

If service n°122 is "available" in EFUST, this file shall be present.

This EF contains the following 5GS location information for non-3GPP access:

-	5G-Globally Unique Temporary Identifier (5G-GUTI);

-	Last visited registered Tracking Area Identity in 5GS (TAI);

-	5GS update status.

| Identifier: '4F02' |  | Structure: transparent |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| SFI: '02' |  |  |  |  |  |  |
| File size: 20 bytes |  |  | Update activity: high |  |  |  |
| Access Conditions: READ PIN UPDATE PIN DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 to 13 | 5G-GUTI for non-3GPP access |  |  | M |  | 13 bytes |
| 14 to19 | Last visited registered TAI in 5GS for non-3GPP access |  |  | M |  | 6 bytes |
| 20 | 5GS update status for non-3GPP access |  |  | M |  | 1 byte |

For content and coding see clause 4.4.11.2 for EF5GS3GPPLOCI.

*** End of change ***