3GPP TSG CT WG1 Meeting #115	C1-191612

# Montreal (Canada), 25 Feb - 1 March 2019	(was C1-191342)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 0915 | rev | 1 | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the REGISTRATION REQUEST msg when the Payload container IE is included |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_Ph1-CT |  |  |  |  |  | Date: |  |  | 2019-02-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The specification allows providing the Payload container IE in several messages (i.e., REGISTRATION REQUEST, DL NAS TRANSPORT or UL NAS TRANSPORT message). For the DL NAS TRANSPORT message and the UL NAS TRANSPORT message, the Payload container IE is accompanied by the Payload container type IE. This allows the AMF to forward the Payload container IE to the correct network entity. However, for the REGISTRATION REQUEST message, there is no Payload container type IE included in the message, and therefore it is unclear clear how to decode the Payload container in this message and to which entity the AMF needs to forward the message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The Payload container type IE is added to the REGISTRATION REQUEST message. Interoperability impact analysis - Network compliant with TS 24.501 v15.2.1 interworking with UE compliant with this CR, and nework compliant with this CR interworking with UE compliant with TS 24.501 v15.2.1: This CR does not result in any interoperability issue as at present the UE can only include the Payload container IE in the REGISTRATION REQUEST message to carry a payload of type "UE policy container" as described in sub-clause 5.5.1.2.2 and 5.5.1.3.2. However, if in a future version of the specification the UE is allowed to provide a Payload container IE with a payload of different type that "UE policy container", a legacy AMF, which does not implements this CR, can forward the Payload container IE to an incorrect network entity. Then, the registration procedure won’t fail but the information conveyed by the Payload container IE may not be taken into account. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong implementation as it is unclear how to decode the Payload container IE included in a REGISTRATION REQUEST message as there is not Payload container type IE included in the message. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.6.1, (new) 8.2.6.17A, 9.11.3.40 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X |  |  |  |  |  |  |  |
| affected: |  |  | X |  |  |  |  |  |  |  |
| (show related CRs) |  |  | X |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** Next change ***

#### 8.2.6.1	Message definition

The REGISTRATION REQUEST message is sent by the UE to the AMF. See table 8.2.6.1.1.

Message type:	REGISTRATION REQUEST

Significance:	dual

Direction:		UE to network

Table 8.2.6.1.1: REGISTRATION REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended Protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Registration request message identity | Message type 9.7 | M | V | 1 |
|  | 5GS registration type | 5GS registration type 9.11.3.7 | M | V | 1/2 |
|  | ngKSI | NAS key set identifier 9.11.3.32 | M | V | 1/2 |
|  | 5GS mobile identity | 5GS mobile identity 9.11.3.4 | M | LV-E | 6-n |
| C- | Non-current native NAS key set identifier | NAS key set identifier 9.11.3.32 | O | TV | 1 |
| 10 | 5GMM capability | 5GMM capability 9.11.3.1 | O | TLV | 3-15 |
| 2E | UE security capability | UE security capability 9.11.3.54 | O | TLV | 4-10 |
| 2F | Requested NSSAI | NSSAI 9.11.3.37 | O | TLV | 4-74 |
| 52 | Last visited registered TAI | 5GS tracking area identity 9.11.3.8 | O | TV | 7 |
| 17 | S1 UE network capability | S1 UE network capability 9.11.3.48 | O | TLV | 4-15 |
| 40 | Uplink data status | Uplink data status 9.11.3.57 | O | TLV | 4-34 |
| 50 | PDU session status | PDU session status 9.11.3.44 | O | TLV | 4-34 |
| B- | MICO indication | MICO indication 9.11.3.31 | O | TV | 1 |
| 2B | UE status | UE status 9.11.3.56 | O | TLV | 3 |
| 77 | Additional GUTI | 5GS mobile identity 9.11.3.4 | O | TLV-E | 14 |
| 25 | Allowed PDU session status | Allowed PDU session status 9.11.3.13 | O | TLV | 4-34 |
| 18 | UE's usage setting | UE's usage setting 9.11.3.55 | O | TLV | 3 |
| 51 | Requested DRX parameters | 5GS DRX parameters 9.11.3.2A | O | TLV | 3 |
| 70 | EPS NAS message container | EPS NAS message container 9.11.3.24 | O | TLV-E | 4-n |
| 74 | LADN indication | LADN indication 9.11.3.29 | O | TLV-E | 3-811 |
| 8- | Payload container type | Payload container type 9.11.3.40 | O | TV | 1 |
| 7B | Payload container | Payload container 9.11.3.39 | O | TLV-E | 4-65538 |
| 9- | Network slicing indication | Network slicing indication 9.11.3.36 | O | TV | 1 |
| 53 | 5GS update type | 5GS update type 9.11.3.9A | O | TLV | 3 |
| 71 | NAS message container | NAS message container 9.11.3.33 | O | TLV-E | 4-n |

*** Next change ***

#### 8.2.6.17A	Payload container type

This IE shall be included if the UE includes the Payload container IE.

*** Next change ***

#### 9.11.3.40	Payload container type

The purpose of the Payload container type information element indicates type of payload included in the payload container information element.

The Payload container information element is coded as shown in figure 9.11.3.40.1 and table 9.11.3.40.1.

The Payload container is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 |  | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Payload container type IEI |  |  |  |  | Payload container type value |  |  |  |  | octet 1 |  |

Figure 9.11.3.40.1: Payload container information element

Table 9.11.3.40.1: Payload container information element

| Payload container type value (octet 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 4 | 3 | 2 | 1 |  |
| 0 | 0 | 0 | 1 | N1 SM information |
| 0 | 0 | 1 | 0 | SMS |
| 0 | 0 | 1 | 1 | LTE Positioning Protocol (LPP) message container |
| 0 | 1 | 0 | 0 | SOR transparent container |
| 0 | 1 | 0 | 1 | UE policy container |
| 0 | 1 | 1 | 0 | UE parameters update transparent container |
| 1 | 1 | 1 | 1 | Multiple payloads |
| All other values are reserved. |  |  |  |  |
| NOTE: Value "Multiple payloads" is used when the Payload container contents in figure 9.11.3.39.1 contains multiple payloads formatted as shown in figure 9.11.3.39.2. |  |  |  |  |