3GPP TSG CT WG1 Meeting #115	C1-191613

# Montreal (Canada), 25 Feb - 1 March 2019	(was C1-191345)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 0917 | rev | 1 | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the length of the IMEISV |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_Ph1-CT |  |  |  |  |  | Date: |  |  | 2019-02-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The specification indicates that the length of the IMEISV in the SECURITY MODE COMPLETE message is 11 octets. However, the above length is incorrect. Note that TS 23.003 (subclause 6.2.2) indicates that the length of IMEISV is as follows: 3 octets for TL plus 9 octets for the identity type and 16 digits (8 digits TAC + 6 digits SNR + 2 digits SVN). After tracking back the CRs which has added or updated the 5G mobile identity IE and this message, one sees that this of having the IMEISV of a length of 11 octets is actually a copy-and-paste mistake done from TS 24.301 (EPS) where the length there it is actually 11 but it has to be 12 for 5GS (i.e., TS 24.501) as the length of the 5G mobile identity IE is 2 octets rather than 1 in TS 24.301 for the Mobile identity IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The length of the IMEISV is corrected to 12 octets. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect length of the IMEISV when coded in the SECURITY MODE COMPLETE message. This leads to not following the IMEISV structure defined in TS 23.003 and also to wrong encoding of the IMEISV as one octet is actually missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.26.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X |  |  |  |  |  |  |  |
| affected: |  |  | X |  |  |  |  |  |  |  |
| (show related CRs) |  |  | X |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** Next change ***

#### 8.2.26.1	Message definition

The SECURITY MODE COMPLETE message is sent by the UE to the AMF in response to a SECURITY MODE COMMAND message. See table 8.2.26.1.1.

Message type:	SECURITY MODE COMPLETE

Significance:	dual

Direction:		UE to network

Table 8.2.26.1.1: SECURITY MODE COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Security mode complete message identity | Message type 9.6 | M | V | 1 |
| 77 | IMEISV | 5G mobile identity 9.11.3.4 | O | TLV-E | 12 |
| 71 | NAS message container | NAS message container 9.11.3.33 | O | TLV-E | 4-n |