3GPP TSG CT WG1 Meeting #115	C1-191347

# Montreal (Canada), 25 Feb - 1 March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 0919 | rev | - | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the Payload container IE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_Ph1-CT |  |  |  |  |  | Date: |  |  | 2019-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The Payloaad container IE sub-clause contains a number of mistakes as follows: - The encoding of the Type of optional IE parameter indicates quote: Type of optional IE (octet xi +2) This field contains the IEI of the optional IE entry and is 1 octet in length. IEI Optional IE name Optional IE reference 12 PDU session ID PDU session identity 2 (see subclause 9.11.3.41) 24 Additional information Additional information (see subclause 9.11.2.1) 58 5GMM cause 5GMM cause (see subclause 9.11.3.2) 37 Back-off timer value GPRS timer 3 (see subclause 9.11.2.5) 59 Old PDU session ID PDU session identity (see subclause 2 9.11.3.41) 8- Request type Request type (see subclause 9.11.3.47) 22 S-NSSAI S-NSSAI (see subclause 9.11.2.8) 25 DNN DNN (see subclause 9.11.3.1A) It is unclear for implementer how to encode the type of optional IE for the case of the Request type as being indicated as “8-“. It should be made clear that this means that all four bits in the another half octet are encoded as zeroes. - The DNN IE is actually specified in the sub-clause 9.11.2.1A and not in 9.11.3.1A. The mistake actually misleads implementers. - The Payload container content parameters description is incorrect as indicates the following, quote: Payload container contents (octet 4 to octet n); max value of 65535 octets 0 1 1 0 UE parameters update transparent container The encoding “0110” was mistakenly pasted when implementing the CR on multiple payloads (CR686). - The figure 9.11.3.39.3 is not correctly formatted. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrections done as per the reson for change. Interoperability impact analysis This CR does not result in any interoperability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Payload container IE contains a number of incorrection which leads to potential wrong implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.11.3.39 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X |  |  |  |  |  |  |  |
| affected: |  |  | X |  |  |  |  |  |  |  |
| (show related CRs) |  |  | x |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** Next change ***

#### 9.11.3.39	Payload container

The purpose of the Payload container information element is to transport one or multiple payloads. If multiple payloads are transported, the associated information of each payload are also transported together with the payload.

The Payload container information element is coded as shown in figure 9.11.3.39.1, figure 9.11.3.39.2, figure 9.11.3.39.3, figure 9.11.3.39.4 and table 9.11.3.39.1.

The Payload container is a type 6 information element with a minimum length of 4 octets and a maximum length of 65538 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Payload container IEI |  |  |  |  |  |  |  |  |  | octet 1 |
| Length of payload container contents |  |  |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  |  |  | octet 3 |
|  |  |  |  |  |  |  |  |  |  | octet 4 |
| Payload container contents |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | octet n |

Figure 9.11.3.39.1: Payload container information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of entries |  |  |  |  |  |  |  |  | octet 4 |  |
| Payload container entry 1 |  |  |  |  |  |  |  |  | octet 5 octet x2 |  |
| Payload container entry 2 |  |  |  |  |  |  |  |  | octet x2+1 octet x3 |  |
| …… |  |  |  |  |  |  |  |  | … |  |
| Payload container entry i |  |  |  |  |  |  |  |  | octet xi +1 octet n |  |

Figure 9.11.3.39.2: Payload container contents with Payload container type "Multiple payloads"

| 8 | 7 | 6 | 5 |  | 4 | 3 | 2 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of Payload container entry |  |  |  |  |  |  |  |  |  | octet xi +1 |  |
| Number of optional IEs |  |  |  |  | Payload container type |  |  |  |  | octet xi +2 |  |
| Optional IE 1 |  |  |  |  |  |  |  |  |  | octet xi +3 octet y2 |  |
| Optional IE 2 |  |  |  |  |  |  |  |  |  | octet y2+1 octet y3 |  |
| … |  |  |  |  |  |  |  |  |  |  |  |
| Optional IE j |  |  |  |  |  |  |  |  |  | octet yj+1 octet z |  |
| Payload container contents |  |  |  |  |  |  |  |  |  | octet z+1 octet n |  |

Figure 9.11.3.39.3: Payload container entry

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of optional IE |  |  |  |  |  |  |  |  | octet xi +2 |  |
| Length of optional IE |  |  |  |  |  |  |  |  | octet xi +3 |  |
| Value of optional IE |  |  |  |  |  |  |  |  | octet xi +4 octet y2 |  |

Figure 9.11.3.39.4: Optional IE

Table 9.11.3.39.1: Payload container information element

| Payload container contents (octet 4 to octet n); max value of 65535 octets |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| If the payload container type is SOR transparent container and is included in the DL NAS TRANSPORT message, the payload container contents are coded the same way as the contents of the SOR transparent container IE (see subclause 9.11.3.51) for SOR data type with value "0" except that the first three octets are not included. If the payload container type is SOR transparent container and is included in the UL NAS TRANSPORT message, the payload container contents are coded the same way as the contents of the SOR transparent container IE (see subclause 9.11.3.51) for SOR data type with value "1" except that the first three octets are not included. If the payload container type is UE parameters update transparent container and is included in the DL NAS TRANSPORT message, the payload container contents are coded the same way as the contents of the UE parameters update transparent container IE (see subclause 9.11.3.53A) for UE parameters update data type with value "0" except that the first three octets are not included. If the payload container type is UE parameters update transparent container and is included in the UL NAS TRANSPORT message, the payload container contents are coded the same way as the contents of the UE parameters update transparent container IE (see subclause 9.11.3.53A) for UE parameters update data type with value "0" except that the first three octets are not included. If the payload container type is multiple payloads, the number of entries field represents the total number of payload container entries, and the payload container contents field is coded as a list of payload container entry according to figure 9.11.3.39.2, with each payload container entry is coded according to figure 9.11.3.39.3 and figure 9.11.3.39.4. |  |  |  |  |  |  |
| The coding of Payload container contents is dependent on the particular application. |  |  |  |  |  |  |
| Payload container entry For each Payload container entry, the Payload container type represents the payload type and the number of optional IEs field represents the total number of optional IEs associated with the payload in Payload container entry. |  |  |  |  |  |  |
| Optional IE For each optional IE is coded according to figure 9.11.3.39.4. Type of optional IE (octet xi +2) This field contains the IEI of the optional IE entry and is 1 octet in length. |  |  |  |  |  |  |
| IEI |  |  | Optional IE name |  |  | Optional IE reference |
| 12 |  |  | PDU session ID |  |  | PDU session identity 2 (see subclause 9.11.3.41) |
| 24 |  |  | Additional information |  |  | Additional information (see subclause 9.11.2.1) |
| 58 |  |  | 5GMM cause |  |  | 5GMM cause (see subclause 9.11.3.2) |
| 37 |  |  | Back-off timer value |  |  | GPRS timer 3 (see subclause 9.11.2.5) |
| 59 |  |  | Old PDU session ID |  |  | PDU session identity (see subclause 2 9.11.3.41) |
| 80 |  |  | Request type |  |  | Request type (see subclause 9.11.3.47) |
| 22 |  |  | S-NSSAI |  |  | S-NSSAI (see subclause 9.11.2.8) |
| 25 |  |  | DNN |  |  | DNN (see subclause 9.11.2.1A) |
| The following optional IE can be included in the payload container entry along with payload container field as shown in figure 9.11.3.39.3 during network initiated NAS transport procedure according to the following inclusion conditions: PDU session ID The AMF shall include this IE when the Payload container type IE of the payload container entry is set to "N1 SM information". Additional information The AMF may include this IE when the Payload container type IE of the payload container entry is set to "LTE Positioning Protocol (LPP) message container". 5GMM cause The AMF shall include this IE when the Payload container IE of the payload container entry contains an uplink payload which was not forwarded. Back-off timer value The AMF shall include this IE when the Payload container IE of the payload container entry contains an uplink 5GSM message which was not forwarded due to DNN based congestion control, S-NSSAI and DNN based congestion control or S-NSSAI only based congestion control. |  |  |  |  |  |  |
| The following optional IE can be included in the payload container entry along with payload container IE as shown in figure 9.11.3.39.3 during UE initiated NAS transport procedure according to the following inclusion conditions: PDU session ID The UE shall include this IE when the Payload container type IE is set to "N1 SM information". Old PDU session ID The UE shall include this IE if the UL NAS TRANSPORT message transports a PDU SESSION ESTABLISHMENT REQUEST message upon receiving the PDU SESSION MODIFICATION COMMAND message with the 5GSM cause IE set to #39 "reactivation requested". Request type The UE shall include this IE when the PDU session ID IE is included and the Payload container IE contains the PDU SESSION ESTABLISHMENT REQUEST message or the PDU SESSION MODIFICATION REQUEST. S-NSSAI The UE may include this IE when the Request type IE is set to "initial request" or "existing PDU session". DNN The UE may include this IE when the Request type IE is set to "initial request" or "existing PDU session". Additional information The UE may include this IE when the Payload container type IE is set to "LTE Positioning Protocol (LPP) message container". |  |  |  |  |  |  |