3GPP TSG CT WG1 Meeting #115	C1-191358

# Montreal (Canada), 25 Feb - 1 March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 0923 | rev | - | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the Operator-defined access category definitions IE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_Ph1-CT |  |  |  |  |  | Date: |  |  | 2019-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The specification indicates that the maximum length of the Operator-defined access category definitions IE is to TBD. Note that the length of the Operator-defined access category definitions IE can be variable as it depends of the number of parameters which are included in this IE. The length is larger that a type 4 IE and can be very large as depends on how many Operator-defined access category definition are included and inside each of this category definition how many Standardized access category are indicated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The Operator-defined access category definitions IE is a type 6 information element with a minimum length of 3 octets. Interoperability impact analysis This CR does not result in any interoperability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to code or decode the Operator-defined access category definitions IE when included in a message as the maximum length is TBD. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.7.1, 8.2.19.1, 9.11.3.38 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X |  |  |  |  |  |  |  |
| affected: |  |  | X |  |  |  |  |  |  |  |
| (show related CRs) |  |  | x |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** Next change ***

#### 8.2.7.1	Message definition

The REGISTRATION ACCEPT message is sent by the AMF to the UE. See table 8.2.7.1.1.

Message type:	REGISTRATION ACCEPT

Significance:	dual

Direction:		network to UE

Table 8.2.7.1.1: REGISTRATION ACCEPT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Registration accept message identity | Message type 9.7 | M | V | 1 |
|  | 5GS registration result | 5GS registration result 9.11.3.6 | M | LV | 2 |
| 77 | 5G-GUTI | 5GS mobile identity 9.11.3.4 | O | TLV-E | 14 |
| 4A | Equivalent PLMNs | PLMN list 9.11.3.45 | O | TLV | 5-47 |
| 54 | TAI list | 5GS tracking area identity list 9.11.3.9 | O | TLV | 9-114 |
| 15 | Allowed NSSAI | NSSAI 9.11.3.37 | O | TLV | 4-74 |
| 11 | Rejected NSSAI | Rejected NSSAI 9.11.3.46 | O | TLV | 4-42 |
| 31 | Configured NSSAI | NSSAI 9.11.3.37 | O | TLV | 4-146 |
| 21 | 5GS network feature support | 5GS network feature support 9.11.3.5 | O | TLV | 3-5 |
| 50 | PDU session status | PDU session status 9.11.3.44 | O | TLV | 4-34 |
| 26 | PDU session reactivation result | PDU session reactivation result 9.11.3.42 | O | TLV | 4-32 |
| 72 | PDU session reactivation result error cause | PDU session reactivation result error cause 9.11.3.43 | O | TLV-E | 5-515 |
| 79 | LADN information | LADN information 9.11.3.30 | O | TLV-E | 12-1715 |
| B- | MICO indication | MICO indication 9.11.3.31 | O | TV | 1 |
| 9- | Network slicing indication | Network slicing indication 9.11.3.36 | O | TV | 1 |
| 27 | Service area list | Service area list 9.11.3.49 | O | TLV | 6-114 |
| 5E | T3512 value | GPRS timer 3 9.11.2.5 | O | TLV | 3 |
| 5D | Non-3GPP de-registration timer value | GPRS timer 2 9.11.2.4 | O | TLV | 3 |
| 16 | T3502 value | GPRS timer 2 9.11.2.4 | O | TLV | 3 |
| 34 | Emergency number list | Emergency number list 9.11.3.23 | O | TLV | 5-50 |
| 7A | Extended emergency number list | Extended emergency number list 9.11.3.26 | O | TLV-E | 7-65538 |
| 73 | SOR transparent container | SOR transparent container 9.11.3.51 | O | TLV-E | 20-2048 |
| 78 | EAP message | EAP message 9.11.2.2 | O | TLV-E | 7-1503 |
| A- | NSSAI inclusion mode | NSSAI inclusion mode 9.11.3.37A | O | TV | 1 |
| 76 | Operator-defined access category definitions | Operator-defined access category definitions 9.11.3.38 | O | TLV-E | 3-n |
| 51 | Negotiated DRX parameters | 5GS DRX parameters 9.11.3.2A | O | TLV | 3 |

*** Next change ***

#### 8.2.19.1	Message definition

The CONFIGURATION UPDATE COMMAND message is sent by the AMF to the UE. See table 8.2.19.1.1.

Message type:	CONFIGURATION UPDATE COMMAND

Significance:	dual

Direction:		network to UE

Table 8.2.19.1.1: CONFIGURATION UPDATE COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Configuration update command message identity | Message type 9.7 | M | V | 1 |
| D- | Configuration update indication | Configuration update indication 9.11.3.18 | O | TV | 1 |
| 77 | 5G-GUTI | 5GS mobile identity 9.11.3.4 | O | TLV-E | 14 |
| 54 | TAI list | 5GS tracking area identity list 9.11.3.9 | O | TLV | 9-114 |
| 15 | Allowed NSSAI | NSSAI 9.11.3.37 | O | TLV | 4-74 |
| 27 | Service area list | Service area list 9.11.3.49 | O | TLV | 6-114 |
| 43 | Full name for network | Network name 9.11.3.35 | O | TLV | 3-n |
| 45 | Short name for network | Network name 9.11.3.35 | O | TLV | 3-n |
| 46 | Local time zone | Time zone 9.11.3.52 | O | TV | 2 |
| 47 | Universal time and local time zone | Time zone and time 9.11.3.53 | O | TV | 8 |
| 49 | Network daylight saving time | Daylight saving time 9.11.3.19 | O | TLV | 3 |
| 79 | LADN information | LADN information 9.11.3.30 | O | TLV-E | 3-1715 |
| B- | MICO indication | MICO indication 9.11.3.31 | O | TV | 1 |
| 9- | Network slicing indication | Network slicing indication 9.11.3.36 | O | TV | 1 |
| 31 | Configured NSSAI | NSSAI 9.11.3.37 | O | TLV | 4-146 |
| 11 | Rejected NSSAI | Rejected NSSAI 9.11.3.46 | O | TLV | 4-42 |
| 76 | Operator-defined access category definitions | Operator-defined access category definitions 9.11.3.38 | O | TLV-E | 3-n |
| F- | SMS indication | SMS indication 9.10.3.50A | O | TV | 1 |

*** Next change ***

#### 9.11.3.38	Operator-defined access category definitions

The purpose of the Operator-defined access category definitions information element is to provide the UE with the operator-defined access category definitions or to delete the operator-defined access category definitions at the UE.

The Operator-defined access category definitions information element is coded as shown in figure 9.11.3.38.1, figure 9.11.3.38.2 and table 9.11.3.38.1.

The Operator-defined access category definitions is a type 6 information element with a minimum length of 3 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operator-defined access category definitions IEI |  |  |  |  |  |  |  | octet 1 |
| Length of operator-defined access category definitions contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| Operator-defined access category definition 1 |  |  |  |  |  |  |  | octet 4 octet a |
| Operator-defined access category definition 2 |  |  |  |  |  |  |  | octet a+1* octet b* |
| … |  |  |  |  |  |  |  | octet b+1* octet g* |
| Operator-defined access category definition 1 n |  |  |  |  |  |  |  | octet g+1* octet h* |

Figure 9.11.3.38.1: Operator-defined access category definitions information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of operator-defined access category definition contents |  |  |  |  |  |  |  | octet 4 |
| Precedence value |  |  |  |  |  |  |  | octet 5 |
| PSAC | 0 Spare | 0 Spare | Operator-defined access category number |  |  |  |  | octet 6 |
| Length of criteria |  |  |  |  |  |  |  | octet 7 |
| Criteria |  |  |  |  |  |  |  | octet 8 octet a-1 |
| 0 Spare | 0 Spare | 0 Spare | Standardized access category |  |  |  |  | octet a* |

Figure 9.11.3.38.2: Operator-defined access category definition

Table 9.11.3.38.1: Operator-defined access category definitions information element

| Value part of the Operator-defined access category definitions information element (octet 3 to h) The value part of the Operator-defined access category definitions information element consists of zero or several operator-defined access category definition fields. Each operator-defined access category definition field is coded as described in figure 9.11.3.38.2. The length of each operator-defined access category definition field is determined by the length of operator-defined access category definition contents field. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operator-defined access category definition (octet 4 to octet a): |  |  |  |  |  |  |  |  |  |
| Precedence value (octet 5) |  |  |  |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | Precedence value 0 |
| to |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  | Precedence value 255 |
| Operator-defined access category number (bits 5 to 1 of octet 6) |  |  |  |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |  |  |  |
| 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |
| 0 | 0 | 0 | 0 | 0 |  |  |  | Access category number 32 |  |
| to |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 1 | 1 | 1 |  |  |  | Access category number 63 |  |
| Presence of standardized access category (PSAC) (bit 8 of octet 6) |  |  |  |  |  |  |  |  |  |
| PSAC field indicates whether the standardized access category field is present or absent. |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |
| 0 |  | Standardized access category field is not included |  |  |  |  |  |  |  |
| 1 |  | Standardized access category field is included |  |  |  |  |  |  |  |
| Length of criteria (octet 7) |  |  |  |  |  |  |  |  |  |
| Length of criteria field indicates binary coded length of the criteria field. |  |  |  |  |  |  |  |  |  |
| Criteria (octets 8 to octet a-1) |  |  |  |  |  |  |  |  |  |
| The criteria field contains one or more criteria components fields. Each criteria component field shall be encoded as a sequence of a one octet criteria type field and zero or more octets criteria value field. The criteria type field shall be transmitted first. |  |  |  |  |  |  |  |  |  |
| Criteria type |  |  |  |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | DNN type |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  | OS id + OS App Id type |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |  | S-NSSAI type |
| All other values are reserved. |  |  |  |  |  |  |  |  |  |
| For "DNN type", the criteria value field shall be encoded as a sequence of one octet DNN length-value pair count field and one or more DNN length-value pair fields. The DNN length-value pair count field indicates the number of included DNN length-value pair fields. Each DNN length-value pair field is coded as a sequence of one octet DNN value length field and a DNN value field. The DNN value length field indicates the length in octets of the DNN value field. The DNN value field contains an APN as specified in 3GPP TS 23.003 [4]. |  |  |  |  |  |  |  |  |  |
| For "OS Id + OS App Id type", the criteria value field shall be encoded as a sequence of one octet app id value count field and one or more app id value fields. The app id value count field indicates the number of included app id value fields. Each app id value field is coded as a sequence of a sixteen octet OS id value field, one octet OS app id value length field and an OS app id value field. The OS app id value length field indicates the length in octets of the OS app id value field. The OS id value field contains a Universally Unique IDentifier (UUID) as specified in IETF RFC 4122 [35A]. The OS app id value field contains an OS specific application identifier. Coding of the OS app id value field is out of scope of the present document. |  |  |  |  |  |  |  |  |  |
| For "S-NSSAI type", the criteria value field shall be encoded as a sequence of one octet S-NSSAI length-value pair count field and one or more S-NSSAI length-value value fields. The S-NSSAI length-value pair count field indicates the number of included S-NSSAI length-value pair fields. Each S-NSSAI length-value pair field is coded as a sequence of one octet S-NSSAI value length field and an S-NSSAI value field. The S-NSSAI value length field indicates the length in octets of the S-NSSAI value field. The S-NSSAI value field contains one octet SST field optionally followed by three octets SD field. The SST field contains a SST. The SD field contains an SD. SST and SD are specified in 3GPP TS 23.003 [4]. |  |  |  |  |  |  |  |  |  |
| Standardized access category (bits 5 to 1 of octet a) |  |  |  |  |  |  |  |  |  |
| Standardized access category field indicates the access category number of the standardized access category that is used in combination with the access identities to determine the establishment cause. |  |  |  |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |  |  |  |
| 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |
| 0 | 0 | 0 | 0 | 0 |  |  |  | Access category number 0 |  |
| to |  |  |  |  |  |  |  |  |  |
| 0 | 0 | 1 | 1 | 1 |  |  |  | Access category number 7 |  |
| All other values are reserved. |  |  |  |  |  |  |  |  |  |

Editor's note:	Whether the 5QI is a suitable access category criteria type is FFS.