3GPP TSG-CT Meeting #83	CP-190204

# Shenzhen, P. R. China; 18th – 19th March 2019						(revision of C4-190597)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.503 | CR | 0149 | rev | 1 | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | API version update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Orange, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-CT |  |  |  |  |  |  | Date: |  | 2019-03-11 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Nudm_SDM API: - Backwards-compatible changes: - CR#0120r1 - CR#0121r1 - CR#0128r1 - CR#0130r1 - CR#0131r1 - CR#0138r1 - CR#0143r4 - CR#0145r2 - Non-backwards-compatible changes: - CR#0147r2 Nudm_UECM API: - Backwards-compatible changes: - CR#0123r1 - CR#0134r2 - CR#0139r1 - CR#0141r1 Nudm_EE API: - Backwards-compatible changes: - CR#0120r1 - CR#0146r1 Rev1: CR#0134 was revised and approved in CT in rev2. CR#0143 was revised and approved in CT in rev4 No impact on the conclusion regarding the API version number |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Nudm_SDM version number is incremented from v1.0.0 to v2.0.0 - Nudm_UECM version number is incremented from v1.0.0 to v1.0.1 - Nudm_EE version number is incremented from v1.0.0 to v1.0.1 - All externalDocs references are updated to: 3GPP TS 29.503 V15.3.0 - API version of Nudm_SDM incremented from v1 to v2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect API version numbers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1, 6.1.3.1, 6.1.3.2.2, 6.1.3.3.2, 6.1.3.4.2, 6.1.3.5.2, 6.1.3.6.2, 6.1.3.7.2, 6.1.3.8.2, 6.1.3.9.2, 6.1.3.10.2, 6.1.3.11.2, 6.1.3.12.2, 6.1.3.13.2, 6.1.3.14.2, 6.1.3.15.2, 6.1.3.16.2, 6.1.3.17.2, 6.1.3.18.2, A.2, A.3, A.4, A.5, A.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 6.1.1	API URI

The Nudm_SDM service shall use the Nudm_SDM API.

The request URI used in HTTP request from the NF service consumer towards the NF service producer shall have the structure defined in subclause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/{apiVersion}/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "nudm-sdm".

-	The {apiVersion} shall be "v2".

-	The <apiSpecificResourceUriPart> shall be set as described in subclause 6.1.3.

* * * Next Change * * * *

#### 6.1.3.1	Overview

Figure 6.1.3.1-1: Resource URI structure of the nudm-sdm API

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource name (Archetype) | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Supi (Document) | /{supi} | GET | Retrieve UE's subscription data |
| Nssai (Document) | /{supi}/nssai | GET | Retrieve the UE's subscribed Network Slice Selection Assistance Information |
| AccessAndMobilitySubscriptionData (Document) | /{supi}/am-data | GET | Retrieve the UE's subscribed Access and Mobility Data |
| SorAck (Document) | /{supi}/am-data/sor-ack | PUT | Providing acknowledgement of Steering of Roaming |
| SmfSelectionSubscriptionData (Document) | /{supi}/smf-select-data | GET | Retrieve the UE's subscribed SMF Selection Data |
| UeContextInSmfData (Document) | /{supi}/ue-context-in-smf-data | GET | Retrieve the UE's Context in SMF Data |
| SessionManagementSubscriptionData (Document) | /{supi}/sm-data | GET | Retrieve the UE's session management subscription data |
| SMSSubscriptionData (Document) | /{supi}/sms-data | GET | Retrieve the UE's SMS subscription data |
| SMSManagementSubscriptionData (Document) | /{supi}/sms-mng-data | GET | Retrieve the UE's SMS management subscription data |
| SdmSubscriptions (Collection) | /{supi}/sdm-subscriptions | POST | Create a subscription |
| Individual subscription (Document) | /{supi}/sdm-subscriptions/{subscriptionId} | DELETE | Delete the subscription identified by {subscriptionId}, i.e. unsubscribe |
| IdTranslationResult (Document) | /{gpsi}/id-translation-result | GET | Retrieve a UE's SUPI |
| UeContextInSmsfData (Document) | /{supi}/ue-context-in-smsf-data | GET | Retrieve the UE's Context in SMSF Data |
| TraceData (Document) | /{supi}/trace-data | GET | Retrieve Trace Configuration Data |
| SharedData (Collection) | /shared-data | GET | Retrieve shared data |
| SharedDataSubscriptions (Collection) | /shared-data-subscriptions | POST | Create a subscription |
| SharedDataIndividual subscription (Document) | /shared-data-subscriptions/{subscriptionId} | DELETE | Delete the subscription identified by {subscriptionId}, i.e. unsubscribe |

* * * Next Change * * * *

##### 6.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/nudm-sdm/{apiVersion}/{supi}/nssai

This resource shall support the resource URI variables defined in table 6.1.3.2.2-1.

Table 6.1.3.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 6.1.1 |
| apiVersion | See subclause 6.1.1 |
| supi | Represents the Subscription Permanent Identifier (see 3GPP TS 23.501 [2] clause 5.9.2) pattern: "(imsi-[0-9]{5,15}¦nai-.+¦.+)" |

* * * Next Change * * * *

##### 6.1.3.3.2	Resource Definition

Resource URI: {apiRoot}/nudm-sdm/{apiVersion}/{supi}/sdm-subscriptions

This resource shall support the resource URI variables defined in table 6.1.3.3.2-1.

Table 6.1.3.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 6.1.1 |
| apiVersion | See subclause 6.1.1 |
| supi | Represents the Subscription Permanent Identifier (see 3GPP TS 23.501 [2] clause 5.9.2) pattern: "(imsi-[0-9]{5,15}¦nai-.+¦.+)" |

* * * Next Change * * * *

##### 6.1.3.4.2	Resource Definition

Resource URI: {apiRoot}/nudm-sdm/{apiVersion}/{supi}/sdm-subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 6.1.3.4.2-1.

Table 6.1.3.4.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 6.1.1 |
| apiVersion | See subclause 6.1.1 |
| supi | Represents the Subscription Permanent Identifier (see 3GPP TS 23.501 [2] clause 5.9.2) pattern: "(imsi-[0-9]{5,15}¦nai-.+¦.+)" |
| subscriptionId | The subscriptionId identifies an individual subscription to notifications. The value is allocated by the UDM during creation of the Subscription resource. |

* * * Next Change * * * *

##### 6.1.3.5.2	Resource Definition

Resource URI: {apiRoot}/nudm-sdm/{apiVersion}/{supi}/am-data

This resource shall support the resource URI variables defined in table 6.1.3.5.2-1.

Table 6.1.3.5.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 6.1.1 |
| apiVersion | See subclause 6.1.1 |
| supi | Represents the Subscription Permanent Identifier (see 3GPP TS 23.501 [2] clause 5.9.2) pattern: "(imsi-[0-9]{5,15}¦nai-.+¦.+)" |

* * * Next Change * * * *

##### 6.1.3.6.2	Resource Definition

Resource URI: {apiRoot}/nudm-sdm/{apiVersion}/{supi}/smf-select-data

This resource shall support the resource URI variables defined in table 6.1.3.6.2-1.

Table 6.1.3.6.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 6.1.1 |
| apiVersion | See subclause 6.1.1 |
| supi | Represents the Subscription Permanent Identifier (see 3GPP TS 23.501 [2] clause 5.9.2) pattern: "(imsi-[0-9]{5,15}¦nai-.+¦.+)" |

* * * Next Change * * * *

##### 6.1.3.7.2	Resource Definition

Resource URI: {apiRoot}/nudm-sdm/{apiVersion}/{supi}/ue-context-in-smf-data

This resource shall support the resource URI variables defined in table 6.1.3.7.2-1.

Table 6.1.3.7.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 6.1.1 |
| apiVersion | See subclause 6.1.1 |
| supi | Represents the Subscription Permanent Identifier (see 3GPP TS 23.501 [2] clause 5.9.2) pattern: "(imsi-[0-9]{5,15}¦nai-.+¦.+)" |

* * * Next Change * * * *

##### 6.1.3.8.2	Resource Definition

Resource URI: {apiRoot}/nudm-sdm/{apiVersion}/{supi}/sm-data

This resource shall support the resource URI variables defined in table 6.1.3.8.2-1.

Table 6.1.3.8.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 6.1.1 |
| apiVersion | See subclause 6.1.1 |
| supi | Represents the Subscription Permanent Identifier (see 3GPP TS 23.501 [2] clause 5.9.2) pattern: "(imsi-[0-9]{5,15}¦nai-.+¦.+)" |

* * * Next Change * * * *

##### 6.1.3.9.2	Resource Definition

Resource URI: {apiRoot}/nudm-sdm/{apiVersion}/{supi}/sms-data

This resource shall support the resource URI variables defined in table 6.1.3.9.2-1.

Table 6.1.3.9.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 6.1.1 |
| apiVersion | See subclause 6.1.1 |
| supi | Represents the Subscription Permanent Identifier (see 3GPP TS 23.501 [2] clause 5.9.2) pattern: "(imsi-[0-9]{5,15}¦nai-.+¦.+)" |

* * * Next Change * * * *

##### 6.1.3.10.2	Resource Definition

Resource URI: {apiRoot}/nudm-sdm/{apiVersion}/{supi}/sms-mng-data

This resource shall support the resource URI variables defined in table 6.1.3.10.2-1.

Table 6.1.3.10.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 6.1.1 |
| apiVersion | See subclause 6.1.1 |
| supi | Represents the Subscription Permanent Identifier (see 3GPP TS 23.501 [2] clause 5.9.2) pattern: "(imsi-[0-9]{5,15}¦nai-.+¦.+)" |

* * * Next Change * * * *

##### 6.1.3.11.2	Resource Definition

Resource URI: {apiRoot}/nudm-sdm/{apiVersion}/{supi}

This resource shall support the resource URI variables defined in table 6.1.3.11.2-1.

Table 6.1.3.11.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 6.1.1 |
| apiVersion | See subclause 6.1.1 |
| supi | Represents the Subscription Permanent Identifier (see 3GPP TS 23.501 [2] clause 5.9.2) pattern: "(imsi-[0-9]{5,15}¦nai-.+)" |

* * * Next Change * * * *

##### 6.1.3.12.2	Resource Definition

Resource URI: {apiRoot}/nudm-sdm/{apiVersion}/{gpsi}/id-translation-result

This resource shall support the resource URI variables defined in table 6.1.3.12.2-1.

Table 6.1.3.12.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 6.1.1 |
| apiVersion | See subclause 6.1.1 |
| gpsi | Represents the Generic Public Subscription Identifier (see 3GPP TS 23.501 [2] subclause 5.9.8) pattern: "(msisdn-[0-9]{5,15}¦extid-[^@]+@[^@]+¦.+)" |

* * * Next Change * * * *

##### 6.1.3.13.2	Resource Definition

Resource URI: {apiRoot}/nudm-sdm/{apiVersion}/{supi}/am-data/sor-ack

This resource shall support the resource URI variables defined in table 6.1.3.13.2-1.

Table 6.1.3.13.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 6.1.1 |
| apiVersion | See subclause 6.1.1 |
| supi | Represents the Subscription Permanent Identifier (see 3GPP TS 23.501 [2] clause 5.9.2) pattern: "(imsi-[0-9]{5,15}¦nai-.+¦.+)" |

* * * Next Change * * * *

##### 6.1.3.14.2	Resource Definition

Resource URI: {apiRoot}/nudm-sdm/{apiVersion}/{supi}/trace-data

This resource shall support the resource URI variables defined in table 6.1.3.14.2-1.

Table 6.1.3.14.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 6.1.1 |
| apiVersion | See subclause 6.1.1 |
| supi | Represents the Subscription Permanent Identifier (see 3GPP TS 23.501 [2] clause 5.9.2) pattern: "(imsi-[0-9]{5,15}¦nai-.+¦.+)" |

* * * Next Change * * * *

##### 6.1.3.15.2	Resource Definition

Resource URI: {apiRoot}/nudm-sdm/{apiVersion}/shared-data

This resource shall support the resource URI variables defined in table 6.1.3.15.2-1.

Table 6.1.3.15.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 6.1.1 |
| apiVersion | See subclause 6.1.1 |

* * * Next Change * * * *

##### 6.1.3.16.2	Resource Definition

Resource URI: {apiRoot}/nudm-sdm/{apiVersion}/shared-data-subscriptions

This resource shall support the resource URI variables defined in table 6.1.3.16.2-1.

Table 6.1.3.16.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 6.1.1 |
| apiVersion | See subclause 6.1.1 |

* * * Next Change * * * *

##### 6.1.3.17.2	Resource Definition

Resource URI: {apiRoot}/nudm-sdm/{apiVersion}/shared-data-subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 6.1.3.17.2-1.

Table 6.1.3.17.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 6.1.1 |
| apiVersion | See subclause 6.1.1 |
| subscriptionId | The subscriptionId identifies an individual subscription to notifications. The value is allocated by the UDM during creation of the Subscription resource. |

* * * Next Change * * * *

##### 6.1.3.18.2	Resource Definition

Resource URI: {apiRoot}/nudm-sdm/{apiVersion}/{supi}/ue-context-in-smsf-data

This resource shall support the resource URI variables defined in table 6.1.3.18.2-1.

Table 6.1.3.18.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 6.1.1 |
| apiVersion | See subclause 6.1.1 |
| supi | Represents the Subscription Permanent Identifier (see 3GPP TS 23.501 [2] clause 5.9.2) pattern: "(imsi-[0-9]{5,15}¦nai-.+¦.+)" |

* * * Next Change * * * *

## A.2	Nudm_SDM API

openapi: 3.0.0

info:

version: '2.0.0'

title: 'Nudm_SDM'

description: 'Nudm Subscriber Data Management Service'

externalDocs:

description: 3GPP TS 29.503 Unified Data Management Services, version 15.3.0

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.503/'

servers:

- url: '{apiRoot}/nudm-sdm/v2'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause subclause 4.4 of 3GPP TS 29.501.

************text not shown for clarity***********

* * * Next Change * * * *

## A.3	Nudm_UECM API

openapi: 3.0.0

info:

version: '1.0.1'

title: 'Nudm_UECM'

description: 'Nudm Context Management Service'

externalDocs:

description: 3GPP TS 29.503 Unified Data Management Services, version 15.3.0

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.503/'

************text not shown for clarity***********

* * * Next Change * * * *

## A.4	Nudm_UEAU API

openapi: 3.0.0

info:

version: '1.0.0'

title: 'NudmUEAU'

description: 'UDM UE Authentication Service'

externalDocs:

description: 3GPP TS 29.503 Unified Data Management Services, version 15.3.0

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.503/'

************text not shown for clarity***********

* * * Next Change * * * *

## A.5	Nudm_EE API

openapi: 3.0.0

info:

version: '1.0.1'

title: 'Nudm_EE'

description: 'Nudm Event Exposure Service'

externalDocs:

description: 3GPP TS 29.503 Unified Data Management Services, version 15.3.0

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.503/'

************text not shown for clarity***********

* * * Next Change * * * *

## A.6	Nudm_PP API

openapi: 3.0.0

info:

version: '1.0.0'

title: 'Nudm_PP'

description: 'Nudm Parameter Provision Service'

externalDocs:

description: 3GPP TS 29.503 Unified Data Management Services, version 15.3.0

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.503/'

************text not shown for clarity***********

* * * End of Change * * * *