3GPP TSG-CT Meeting #84	CP-191069

# Newport Beach, US; 03rd – 04th June 2019						  rev of C1-192684, C1-192133

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 27.007 | CR | 0652 | rev | 2 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Indicating to the TE whether the UE is EPS attached for access to RLOS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung R&D Institute UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Samsung R&D Institute UK |  |  |  |  |  |  |  |  |  |
| Work item code: | PARLOS |  |  |  |  |  | Date: |  |  | 2019-05-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In E-UTRAN, a UE (in limited service state) can be attached for access to RLOS which is different than being attached for emergency bearer services. A UE performs a EPS attach for RLOS upon explicit request from the user. +CREG and +CEREG should be updated to allow the MT to indicate to the TE that the UE is attached for access to RLOS. Rev 1 was (pre-)agreed at CT1#116, however a corrupted version was uploaded to the 3GPP server. Rev 2 provides a fixed version. No technical changes have made between Rev 1 and Rev 2. NOTE: The change between the original version of the CR and Rev 1 was to replace "MS" with "MT" in NOTE 2 and NOTE 2a in each of the subclauses. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update +CREG, +CGREG, +CEREG and +C5GREG to add a new value for <stat> to indicate "attached for access to RLOS" and indicate usage for CREG and CEREG only when <AcT> is E-UTRAN (i.e. WB-S1 and not NB-S1) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Application will not know the the UE is attached for access to RLOS in limited service state. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2, 10.1.20, 10.1.22, 10.1.47 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | C1-192684 (rev-1), C1-192133 |  |  |  |  |  |  |  |  |

## 7.2	Network registration +CREG

Table 35: +CREG parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| +CREG=[<n>] | +CME ERROR: <err> |
| +CREG? | +CREG: <n>,<stat>[,[<lac>],[<ci>],[<AcT>][,<cause_type>,<reject_cause>]] |
| +CREG=? | +CREG: (list of supported <n>s) |

Description

Set command controls the presentation of an unsolicited result code +CREG: <stat> when <n>=1 and there is a change in the MT’s circuit mode network registration status in GERAN/UTRAN/E-UTRAN, or unsolicited result code +CREG: <stat>[,[<lac>],[<ci>],[<AcT>]] when <n>=2 and there is a change of the network cell in GERAN/UTRAN/E-UTRAN. The parameters <AcT>, <lac> and <ci> are sent only if available. The value <n>=3 further extends the unsolicited result code with [,<cause_type>,<reject_cause>], when available, when the value of <stat> changes.

NOTE 1:	If the MT also supports one or more of the GPRS services, EPS services or 5G services, the +CGREG command and +CGREG: result codes, the +CEREG command and +CEREG: result codes and the +C5GREG command and +C5GREG: result codes apply to the registration status and location information for those services.

Read command returns the status of result code presentation and an integer <stat> which shows whether the network has currently indicated the registration of the MT. Location information elements <lac>, <ci> and <AcT>, if available, are returned only when <n>=2 and MT is registered in the network. The parameters [,<cause_type>,<reject_cause>], if available, are returned when <n>=3. Refer subclause 9.2 for possible <err> values.

Test command returns values supported as a compound value.

Defined values

<n>: integer type

0	disable network registration unsolicited result code

1	enable network registration unsolicited result code +CREG: <stat>

2	enable network registration and location information unsolicited result code +CREG: <stat>[,[<lac>],[<ci>],[<AcT>]]

3	enable network registration, location information and cause value information unsolicited result code +CREG: <stat>[,[<lac>],[<ci>],[<AcT>][,<cause_type>,<reject_cause>]]

<stat>: integer type; circuit mode registration status.

0	not registered, MT is not currently searching a new operator to register to

1	registered, home network

2	not registered, but MT is currently searching a new operator to register to

3	registration denied

4	unknown (e.g. out of GERAN/UTRAN/E-UTRAN coverage)

5	registered, roaming

6	registered for "SMS only", home network (applicable only when <AcT> indicates E-UTRAN)

7	registered for "SMS only", roaming (applicable only when <AcT> indicates E-UTRAN)

8	attached for emergency bearer services only (see NOTE 2) (not applicable)

9	registered for "CSFB not preferred", home network (applicable only when <AcT> indicates E-UTRAN)

10	registered for "CSFB not preferred", roaming (applicable only when <AcT> indicates E-UTRAN)

11	attached for access to RLOS (see NOTE 2a) (applicable only when <AcT> indicates E-UTRAN)

NOTE 2:	3GPP TS 24.008 [8] and 3GPP TS 24.301 [83] specify the condition when the MT is considered as attached for emergency bearer services.

NOTE 2a:	3GPP TS 24.301 [83] specifies the condition when the MT is considered as attached for access to RLOS.

<lac>: string type; two byte location area code (when <AcT> indicates value 0 to 6), or tracking area code (when <AcT> indicates value 7). In hexadecimal format (e.g. "00C3" equals 195 in decimal).

<ci>: string type; four byte GERAN/UTRAN/E-UTRAN cell ID in hexadecimal format.

<AcT>: integer type; access technology of the serving cell.

0	GSM

1	GSM Compact

2	UTRAN

3	GSM w/EGPRS (see NOTE 3)

4	UTRAN w/HSDPA (see NOTE 4)

5	UTRAN w/HSUPA (see NOTE 4)

6	UTRAN w/HSDPA and HSUPA (see NOTE 4)

7	E-UTRAN

8	EC-GSM-IoT (A/Gb mode) (see NOTE 5)

9	E-UTRAN (NB-S1 mode) (see NOTE 6)

10	E-UTRA connected to a 5GCN (see NOTE 7) (not applicable)

11	NR connected to a 5GCN (see NOTE 7) (not applicable)

12	NG-RAN (not applicable)

13	E-UTRA-NR dual connectivity (see NOTE 8)

NOTE 3:	3GPP TS 44.018 [156] specifies the System Information messages which give the information about whether the serving cell supports EGPRS.

NOTE 4:	3GPP TS 25.331 [74] specifies the System Information blocks which give the information about whether the serving cell supports HSDPA or HSUPA.

NOTE 5:	3GPP TS 44.018 [156] specifies the EC-SCH INFORMATION message which, if present, indicates that the serving cell supports EC-GSM-IoT.

NOTE 6:	3GPP TS 36.331 [86] specifies the System Information blocks which give the information about whether the serving cell supports NB-IoT, which corresponds to E-UTRAN (NB-S1 mode).

NOTE 7:	3GPP TS 38.331 [160] specifies the information which, if present, indicates that the serving cell is connected to a 5GCN.

NOTE 8:	3GPP TS 38.331 [160] specifies the information which, if present, indicates that the serving cell is supporting dual connectivity of E-UTRA with NR and is connected to an EPS core.

<cause_type>: integer type; indicates the type of <reject_cause>.

0	Indicates that <reject_cause> contains an MM cause value, see 3GPP TS 24.008 [8] Annex G.

1	Indicates that <reject_cause> contains a manufacturer specific cause.

<reject_cause>: integer type; contains the cause of the failed registration. The value is of type as defined by <cause_type>.

Implementation

Optional.

This command is not applicable to UEs in NG-RAN.

***** Next change *****

### 10.1.20	GPRS network registration status +CGREG

Table 128: +CGREG parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| +CGREG=[<n>] | +CME ERROR: <err> |
| +CGREG? | when <n>=0, 1, 2 or 3 and command successful: +CGREG: <n>,<stat>[,[<lac>],[<ci>],[<AcT>],[<rac>][,<cause_type>,<reject_cause>]] when <n>=4 or 5 and command successful: +CGREG: <n>,<stat>[,[<lac>],[<ci>],[<AcT>],[<rac>][,[<cause_type>],[<reject_cause>][,[<Active-Time>],[<Periodic-RAU>],[<GPRS-READY-timer>]]]] |
| +CGREG=? | +CGREG: (list of supported <n>s) |

Description

The set command controls the presentation of an unsolicited result code +CGREG: <stat> when <n>=1 and there is a change in the MT's GPRS network registration status in GERAN/UTRAN, or unsolicited result code +CGREG: <stat>[,[<lac>],[<ci>],[<AcT>],[<rac>]] when <n>=2 and there is a change of the network cell in GERAN/UTRAN. The parameters <AcT>, <lac>, <rac> and <ci> are provided only if available. The value <n>=3 further extends the unsolicited result code with [,<cause_type>,<reject_cause>], when available, when the value of <stat> changes.

If the UE wants to apply PSM for reducing its power consumption, see +CPSMS command and 3GPP TS 23.682 [149], the set command controls the presentation of an unsolicited result code +CGREG: <stat>[,[<lac>],[<ci>],[<AcT>],[<rac>][,[<cause_type>],[<reject_cause>][,[<Active-Time>],[<Periodic-RAU>],[<GPRS-READY-timer>]]]]. When <n>=4 the unsolicited result code will provide the UE with additional information for the Active Time value, the extended periodic RAU value and the GPRS READY timer value if there is a change of the network cell in GERAN/UTRAN. The value <n>=5 further enhances the unsolicited result code with <cause_type> and <reject_cause> when the value of <stat> changes. The parameters <AcT>, <lac>, <rac>, <ci>, <cause_type>, <reject_cause>, <Active-Time>, <Periodic-RAU> and <GPRS-READY-timer> are provided only if available.

Refer subclause 9.2 for possible <err> values.

NOTE 1:	If the GPRS MT also supports one or more of the circuit mode services in GERAN/UTRAN, EPS services in E-UTRAN or 5G services in NG-RAN, the +CREG command and +CREG: result codes, the +CEREG command and +CEREG: result codes and the +C5GREG command and +C5GREG: result codes apply to the registration status and location information for those services.

The read command returns the status of result code presentation and an integer <stat> which shows whether the network has currently indicated the registration of the MT. Location information elements <lac>, <ci>, <AcT> and <rac>, if available, are returned only when <n>=2 and MT is registered in the network. The parameters [,<cause_type>,<reject_cause>], if available, are returned when <n>=3.

Test command returns values supported as a compound value.

Defined values

<n>: integer type

0	disable network registration unsolicited result code

1	enable network registration unsolicited result code +CGREG: <stat>

2	enable network registration and location information unsolicited result code +CGREG: <stat>[,<[lac>,]<[ci>],[<AcT>],[<rac>]]

3	enable network registration, location information and GMM cause value information unsolicited result code +CGREG: <stat>[,[<lac>],[<ci>],[<AcT>],[<rac>][,<cause_type>,<reject_cause>]]

4	For a UE that wants to apply PSM, enable network registration and location information unsolicited result code +CGREG: <stat>[,[<lac>],[<ci>],[<AcT>],[<rac>][,,[,[<Active-Time>],[<Periodic-RAU>],[<GPRS-READY-timer>]]]]

5	For a UE that wants to apply PSM, enable network registration, location information and GMM cause value information unsolicited result code +CGREG: <stat>[,[<lac>],[<ci>],[<AcT>],[<rac>][,[<cause_type>],[<reject_cause>][,[<Active-Time>],[<Periodic-RAU>],[<GPRS-READY-timer>]]]]

<stat>: integer type; indicates the GPRS registration status.

0	not registered, MT is not currently searching an operator to register to

1	registered, home network

2	not registered, but MT is currently trying to attach or searching an operator to register to

3	registration denied

4	unknown (e.g. out of GERAN/UTRAN coverage)

5	registered, roaming

6	registered for "SMS only", home network (not applicable)

7	registered for "SMS only", roaming (not applicable)

8	attached for emergency bearer services only (see NOTE 2) (applicable only when <AcT> indicates 2,4,5,6)

9	registered for "CSFB not preferred", home network (not applicable)

10	registered for "CSFB not preferred", roaming (not applicable)

11	attached for access to RLOS (See NOTE 2a) (not applicable)

NOTE 2:	3GPP TS 24.008 [8] and 3GPP TS 24.301 [83] specify the condition when the MT is considered as attached for emergency bearer services.

NOTE 2a:	3GPP TS 24.301 [83] specifies the condition when the MT is considered as attached for access to RLOS.

<lac>: string type; two byte location area code in hexadecimal format (e.g. "00C3" equals 195 in decimal).

<ci>: string type; four byte GERAN/UTRAN cell ID in hexadecimal format.

<AcT>: integer type; indicates the access technology of the serving cell.

0	GSM

1	GSM Compact

2	UTRAN

3	GSM w/EGPRS (see NOTE 3)

4	UTRAN w/HSDPA (see NOTE 4)

5	UTRAN w/HSUPA (see NOTE 4)

6	UTRAN w/HSDPA and HSUPA (see NOTE 4)

7	E-UTRAN (not applicable)

8	EC-GSM-IoT (A/Gb mode) (see NOTE 5)

9	E-UTRAN (NB-S1 mode) (see NOTE 6) (not applicable)

10	E-UTRA connected to a 5GCN (see NOTE 7) (not applicable)

11	NR connected to a 5GCN (see NOTE 7) (not applicable)

12	NG-RAN (not applicable)

13	E-UTRA-NR dual connectivity (see NOTE 8) (not applicable)

NOTE 3:	3GPP TS 44.018 [156] specifies the System Information messages which give the information about whether the serving cell supports EGPRS.

NOTE 4:	3GPP TS 25.331 [74] specifies the System Information blocks which give the information about whether the serving cell supports HSDPA or HSUPA.

NOTE 5:	3GPP TS 44.018 [156] specifies the EC-SCH INFORMATION message which, if present, indicates that the serving cell supports EC-GSM-IoT.

NOTE 6:	3GPP TS 36.331 [86] specifies the System Information blocks which give the information about whether the serving cell supports NB-IoT, which corresponds to E-UTRAN (NB-S1 mode).

NOTE 7:	3GPP TS 38.331 [160] specifies the information which, if present, indicates that the serving cell is connected to a 5GCN.

NOTE 8:	3GPP TS 38.331 [160] specifies the information which, if present, indicates that the serving cell is supporting dual connectivity of E-UTRA with NR and is connected to an EPS core.

<rac>: string type; one byte routing area code in hexadecimal format.

<cause_type>: integer type; indicates the type of <reject_cause>.

0	Indicates that <reject_cause> contains a GMM cause value, see 3GPP TS 24.008 [8] Annex G.

1	Indicates that <reject_cause> contains a manufacturer-specific cause.

<reject_cause>: integer type; contains the cause of the failed registration. The value is of type as defined by <cause_type>.

<Active-Time>: string type; one byte in an 8 bit format. Indicates the Active Time value (T3324) allocated to the UE in GERAN/UTRAN. The Active Time value is coded as one byte (octet 3) of the GPRS Timer 2 information element coded as bit format (e.g. "00100100" equals 4 minutes). For the coding and the value range, see the GPRS Timer 2 IE in 3GPP TS 24.008 [8] Table 10.5.163/3GPP TS 24.008. See also 3GPP TS 23.682 [149] and 3GPP TS 23.060 [47].

<Periodic-RAU>: string type; one byte in an 8 bit format. Indicates the extended periodic RAU value (T3312) allocated to the UE in GERAN/UTRAN. The extended periodic RAU value s coded as one byte (octet 3) of the GPRS Timer 3 information element coded as bit format (e.g. "01000111" equals 70 hours). For the coding and the value range, see the GPRS Timer 3 IE in 3GPP TS 24.008 [8] Table 10.5.163a/3GPP TS 24.008. See also 3GPP TS 23.682 [149] and 3GPP TS 23.060 [47].

<GPRS-READY-timer>: string type; one byte in an 8 bit format. Indicates the GPRS READY timer value (T3314) allocated to the UE in GERAN/UTRAN. The GPRS READY timer value is coded as one byte (octet 2) of the GPRS Timer information element coded as bit format (e.g. "01000011" equals 3 decihours or 18 minutes). For the coding and the value range, see the GPRS Timer IE in 3GPP TS 24.008 [8] Table 10.5.172/3GPP TS 24.008. See also 3GPP TS 23.060 [47].

Implementation

Optional.

This command is not applicable to UEs in E-UTRAN or NG-RAN.

***** Next change *****

### 10.1.22	EPS network registration status +CEREG

Table 10.1.22-1: +CEREG parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| +CEREG=[<n>] | +CME ERROR: <err> |
| +CEREG? | when <n>=0, 1, 2 or 3 and command successful: +CEREG: <n>,<stat>[,[<tac>],[<ci>],[<AcT>[,<cause_type>,<reject_cause>]]] when <n>=4 or 5 and command successful: +CEREG: <n>,<stat>[,[<tac>],[<ci>],[<AcT>][,[<cause_type>],[<reject_cause>][,[<Active-Time>],[<Periodic-TAU>]]]] |
| +CEREG=? | +CEREG: (list of supported <n>s) |

Description

The set command controls the presentation of an unsolicited result code +CEREG: <stat> when <n>=1 and there is a change in the MT's EPS network registration status in E-UTRAN, or unsolicited result code +CEREG: <stat>[,[<tac>],[<ci>],[<AcT>]] when <n>=2 and there is a change of the network cell in E-UTRAN. The parameters <AcT>, <tac> and <ci> are provided only if available. The value <n>=3 further extends the unsolicited result code with [,<cause_type>,<reject_cause>], when available, when the value of <stat> changes.

If the UE wants to apply PSM for reducing its power consumption, see +CPSMS command and 3GPP TS 23.682 [149], the set command controls the presentation of an unsolicited result code +CEREG: <stat>[,[<tac>],[<ci>],[<AcT>][,[<cause_type>],[<reject_cause>][,[<Active-Time>],[<Periodic-TAU>]]]]. When <n>=4 the unsolicited result code will provide the UE with additional information for the Active Time value and the extended periodic TAU value if there is a change of the network cell in E-UTRAN. The value <n>=5 further enhances the unsolicited result code with <cause_type> and <reject_cause> when the value of <stat> changes. The parameters <AcT>, <tac>, <ci>, <cause_type>, <reject_cause>, <Active-Time> and <Periodic-TAU> are provided only if available.

Refer subclause 9.2 for possible <err> values.

NOTE 1:	If the EPS MT in GERAN/UTRAN/E-UTRAN also supports one or more of the circuit mode services, GPRS services or 5G services, the +CREG command and +CREG: result codes, the +CGREG command and +CGREG: result codes and the +C5GREG command and +C5GREG: result codes apply to the registration status and location information for those services.

The read command returns the status of result code presentation and an integer <stat> which shows whether the network has currently indicated the registration of the MT. Location information elements <tac>, <ci> and <AcT>, if available, are returned only when <n>=2 and MT is registered in the network. The parameters [,<cause_type>,<reject_cause>], if available, are returned when <n>=3.

Test command returns values supported as a compound value.

Defined values

<n>: integer type

0	disable network registration unsolicited result code

1	enable network registration unsolicited result code +CEREG: <stat>

2	enable network registration and location information unsolicited result code +CEREG: <stat>[,[<tac>],[<ci>],[<AcT>]]

3	enable network registration, location information and EMM cause value information unsolicited result code +CEREG: <stat>[,[<tac>],[<ci>],[<AcT>][,<cause_type>,<reject_cause>]]

4	For a UE that wants to apply PSM, enable network registration and location information unsolicited result code +CEREG: <stat>[,[<tac>],[<ci>],[<AcT>][,,[,[<Active-Time>],[<Periodic-TAU>]]]]

5	For a UE that wants to apply PSM, enable network registration, location information and EMM cause value information unsolicited result code +CEREG: <stat>[,[<tac>],[<ci>],[<AcT>][,[<cause_type>],[<reject_cause>][,[<Active-Time>],[<Periodic-TAU>]]]]

<stat>: integer type; indicates the EPS registration status.

0	not registered, MT is not currently searching an operator to register to

1	registered, home network

2	not registered, but MT is currently trying to attach or searching an operator to register to

3	registration denied

4	unknown (e.g. out of E-UTRAN coverage)

5	registered, roaming

6	registered for "SMS only", home network (not applicable)

7	registered for "SMS only", roaming (not applicable)

8	attached for emergency bearer services only (See NOTE 2)

9	registered for "CSFB not preferred", home network (not applicable)

10	registered for "CSFB not preferred", roaming (not applicable)

11	attached for access to RLOS (See NOTE 2a) (applicable only when <AcT> indicates E-UTRAN)

NOTE 2:	3GPP TS 24.008 [8] and 3GPP TS 24.301 [83] specify the condition when the MT is considered as attached for emergency bearer services.

NOTE 2a:	3GPP TS 24.301 [83] specifies the condition when the MT is considered as attached for access to RLOS.

<tac>: string type; two byte tracking area code in hexadecimal format (e.g. "00C3" equals 195 in decimal).

<ci>: string type; four byte E-UTRAN cell ID in hexadecimal format.

<AcT>: integer type; indicates the access technology of the serving cell.

0	GSM (not applicable)

1	GSM Compact (not applicable)

2	UTRAN (not applicable)

3	GSM w/EGPRS (see NOTE 3) (not applicable)

4	UTRAN w/HSDPA (see NOTE 4) (not applicable)

5	UTRAN w/HSUPA (see NOTE 4) (not applicable)

6	UTRAN w/HSDPA and HSUPA (see NOTE 4) (not applicable)

7	E-UTRAN

8	EC-GSM-IoT (A/Gb mode) (see NOTE 5) (not applicable)

9	E-UTRAN (NB-S1 mode) (see NOTE 6)

10	E-UTRA connected to a 5GCN (see NOTE 7) (not applicable)

11	NR connected to a 5G CN (see NOTE 7) (not applicable)

12	NG-RAN (not applicable)

13	E-UTRA-NR dual connectivity (see NOTE 8)

NOTE 3:	3GPP TS 44.018 [156] specifies the System Information messages which give the information about whether the serving cell supports EGPRS.

NOTE 4:	3GPP TS 25.331 [74] specifies the System Information blocks which give the information about whether the serving cell supports HSDPA or HSUPA.

NOTE 5:	3GPP TS 44.018 [156] specifies the EC-SCH INFORMATION message which, if present, indicates that the serving cell supports EC-GSM-IoT.

NOTE 6:	3GPP TS 36.331 [86] specifies the System Information blocks which give the information about whether the serving cell supports NB-IoT, which corresponds to E-UTRAN (NB-S1 mode).

NOTE 7:	3GPP TS 38.331 [160] specifies the information which, if present, indicates that the serving cell is connected to a 5GCN.

NOTE 8:	3GPP TS 38.331 [160] specifies the information which, if present, indicates that the serving cell is supporting dual connectivity of E-UTRA with NR and is connected to an EPS core.

<cause_type>: integer type; indicates the type of <reject_cause>.

0	Indicates that <reject_cause> contains an EMM cause value, see 3GPP TS 24.301 [83] Annex A.

1	Indicates that <reject_cause> contains a manufacturer-specific cause.

<reject_cause>: integer type; contains the cause of the failed registration. The value is of type as defined by <cause_type>.

<Active-Time>: string type; one byte in an 8 bit format. Indicates the Active Time value (T3324) allocated to the UE in E-UTRAN. The Active Time value is coded as one byte (octet 3) of the GPRS Timer 2 information element coded as bit format (e.g. "00100100" equals 4 minutes). For the coding and the value range, see the GPRS Timer 2 IE in 3GPP TS 24.008 [8] Table 10.5.163/3GPP TS 24.008. See also 3GPP TS 23.682 [149] and 3GPP TS 23.401 [82].

<Periodic-TAU>: string type; one byte in an 8 bit format. Indicates the extended periodic TAU value (T3412) allocated to the UE in E-UTRAN. The extended periodic TAU value is coded as one byte (octet 3) of the GPRS Timer 3 information element coded as bit format (e.g. "01000111" equals 70 hours). For the coding and the value range, see the GPRS Timer 3 IE in 3GPP TS 24.008 [8] Table 10.5.163a/3GPP TS 24.008. See also 3GPP TS 23.682 [149] and 3GPP TS 23.401 [82].

Implementation

Optional.

This command is only applicable to UEs in E-UTRAN.

***** Next change *****

### 10.1.47	5GS network registration status +C5GREG

Table 10.1.47-1: +C5GREG parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| +C5GREG=[<n>] | +CME ERROR: <err> |
| +C5GREG? | when <n>=0, 1, 2 or 3 and command successful: +C5GREG: <n>,<stat>[,[<tac>],[<ci>],[<AcT>],[<Allowed_NSSAI_length>],[<Allowed_NSSAI>][,<cause_type>,<reject_cause>]] |
| +C5GREG=? | +C5GREG: (list of supported <n>s) |

Description

The set command controls the presentation of an unsolicited result code +C5GREG: <stat> when <n>=1 and there is a change in the MT's network registration status in 5GS, or unsolicited result code +C5GREG: <stat>[,[<tac>],[<ci>],[<AcT>],[<Allowed_NSSAI_length>],[<Allowed_NSSAI>]] when <n>=2 and there is a change of the network cell in 5GS or the network provided an Allowed NSSAI. The parameters <AcT>, <tac>, <ci>, <Allowed_NSSAI_length> and <Allowed_NSSAI> are provided only if available. The value <n>=3 further extends the unsolicited result code with [,<cause_type>,<reject_cause>], when available, when the value of <stat> changes.

Refer subclause 9.2 for possible <err> values.

NOTE 1:	If the 5G MT in GERAN/UTRAN/E-UTRAN also supports one or more of the circuit mode services, GPRS services or EPS services, the +CREG command and +CREG: result codes, the +CGREG command and +CGREG: result codes and the +CEREG command and +CEREG: result codes apply to the registration status and location information for those services.

The read command returns the status of result code presentation and an integer <stat> which shows whether the network has currently indicated the registration of the MT. Location information elements <tac>, <ci> and <AcT>, and parameters <Allowed_NSSAI_length>, <Allowed_NSSAI>, if available, are returned only when <n>=2 and MT is registered in the network. The parameters [,<cause_type>,<reject_cause>], if available, are returned when <n>=3.

Test command returns values supported as a compound value.

Defined values

<n>: integer type

0	disable network registration unsolicited result code

1	enable network registration unsolicited result code +C5GREG: <stat>

2	enable network registration and location information unsolicited result code +C5GREG: <stat>[,[<tac>],[<ci>],[<AcT>],[<Allowed_NSSAI_length>],[<Allowed_NSSAI>]]

3	enable network registration, location information and 5GMM cause value information unsolicited result code +C5GREG: <stat>[,[<tac>],[<ci>],[<AcT>],[<Allowed_NSSAI_length>],[<Allowed_NSSAI>][,<cause_type>,<reject_cause>]]

<stat>: integer type; indicates the NR registration status.

0	not registered, MT is not currently searching an operator to register to

1	registered, home network

2	not registered, but MT is currently trying to attach or searching an operator to register to

3	registration denied

4	unknown (e.g. out of NR coverage)

5	registered, roaming

6	registered for "SMS only", home network (not applicable)

7	registered for "SMS only", roaming (not applicable)

8	registered for emergency services only (See NOTE 2)

9	registered for "CSFB not preferred", home network (not applicable)

10	registered for "CSFB not preferred", roaming (not applicable)

11	attached for access to RLOS (See NOTE 2a) (not applicable)

NOTE 2:	3GPP TS 24.501 [161] specifies the condition when the MT is considered as registered for emergency services.

NOTE 2a:	3GPP TS 24.301 [83] specifies the condition when the MT is considered as attached for access to RLOS.

<tac>: string type; three byte tracking area code in hexadecimal format (e.g. "0000C3" equals 195 in decimal).

<ci>: string type; five byte NR cell ID in hexadecimal format.

<Allowed_NSSAI_length>: integer type; indicates the number of octets of the <Allowed_NSSAI> information element.

<Allowed_NSSAI>: string type in hexadecimal format. Dependent of the form, the string can be separated by dot(s), semicolon(s) and colon(s). This parameter indicates the list of allowed S-NSSAIs received from the network. The <Allowed_NSSAI> is coded as a list of <S-NSSAI>s separated by colons. Refer parameter <S-NSSAI> in subclause 10.1.1. This parameter shall not be subject to conventional character conversion as per +CSCS.

<AcT>: integer type; indicates the access technology of the serving cell.

0	GSM (not applicable)

1	GSM Compact (not applicable)

2	UTRAN (not applicable)

3	GSM w/EGPRS (see NOTE 3) (not applicable)

4	UTRAN w/HSDPA (see NOTE 4) (not applicable)

5	UTRAN w/HSUPA (see NOTE 4) (not applicable)

6	UTRAN w/HSDPA and HSUPA (see NOTE 4) (not applicable)

7	E-UTRAN (not applicable)

8	EC-GSM-IoT (A/Gb mode) (see NOTE 5) (not applicable)

9	E-UTRAN (NB-S1 mode) (see NOTE 6) (not applicable)

10	E-UTRA connected to a 5GCN (see NOTE 7)

11	NR connected to a 5GCN (see NOTE 7)

12	NG-RAN (not applicable)

13	E-UTRA-NR dual connectivity (see NOTE 8) (not applicable)

NOTE 3:	3GPP TS 44.018 [156] specifies the System Information messages which give the information about whether the serving cell supports EGPRS.

NOTE 4:	3GPP TS 25.331 [74] specifies the System Information blocks which give the information about whether the serving cell supports HSDPA or HSUPA.

NOTE 5:	3GPP TS 44.018 [156] specifies the EC-SCH INFORMATION message which, if present, indicates that the serving cell supports EC-GSM-IoT.

NOTE 6:	3GPP TS 36.331 [86] specifies the System Information blocks which give the information about whether the serving cell supports NB-IoT, which corresponds to E-UTRAN (NB-S1 mode).

NOTE 7:	3GPP TS 38.331 [160] specifies the information which, if present, indicates that the serving cell is connected to a 5GCN.

NOTE 8:	3GPP TS 38.331 [160] specifies the information which, if present, indicates that the serving cell is supporting dual connectivity of E-UTRA with NR and is connected to an EPS core.

<cause_type>: integer type; indicates the type of <reject_cause>.

0	Indicates that <reject_cause> contains an EMM cause value, see 3GPP TS 24.301 [83] Annex A.

1	Indicates that <reject_cause> contains a manufacturer-specific cause.

<reject_cause>: integer type; contains the cause of the failed registration. The value is of type as defined by <cause_type>.

Implementation

Optional. This command is only applicable to UEs supporting 5GS.

***** End changes *****