3GPP TSG-CT Meeting #84	CP-191163

# Newport Beach California, United States, 3rd Jun 2019 - 3rd Jun 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 1130 | rev | 3 | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | User plane CIoT 5GS optimization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | vivo, InterDigital |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2019-05-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | S2-1906178 introduces 5GS UP optimization. This contribution proposes to implement the corresponding SA2 requirement into stage 3 specificition. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the following changes when user plane CIoT 5GS optimization is used: 1. how the UE suspend and resume the N1 NAS signalling connection; and 2. how the NW suspend and resume the N1 NAS signalling connection; and |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of user plane CIoT 5GS optimization |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS23.501 CR1186 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: 1. add InterDigitalas co-source. 2. add linkage to SA2 CR. 3. tick the UE and CN box. 4. add the handling of pending NAS message when resume succeed. 5. add the UE enters 5GMM-IDLE mode when resume fail. 6. add three editor’s notes. 7. correction of N1 NAS signnaling. Rev2: Rewording the 2nd editor’s note. Rev3: Correct the affected SA2 CR number in the coverpage. |  |  |  |  |  |  |  |  |

***** Next change *****

5.3.1.x	Suspend and resume of the N1 NAS signalling connection

Suspend of the N1 NAS signalling connection can be initiated by the network in 5GMM-CONNECTED mode when user plane CIoT 5GS optimization is used. Resume of the suspended N1 NAS signalling connection is initiated by the UE.

In the UE, when user plane CIoT 5GS optimization is used:

-	Upon indication from the lower layers that the RRC connection has been suspended, the UE shall enter 5GMM-IDLE mode with suspend indication, shall not consider the N1 NAS signalling connection released and shall not consider the secure exchange of NAS messages terminated (see subclause 4.4.2.5 and 4.4.5).

-	Upon trigger of a procedure using an initial NAS message when in 5GMM-IDLE mode with suspend indication, the UE shall:

i)	if the initial NAS message is a REGISTRATION REQUEST message with the NG-RAN-RCU bit of the 5GS update type IE set to "NG-RAN radio capability update needed", enter 5GMM-IDLE mode without suspend indication and proceed with the registration procedure; or

ii)	otherwise, request the lower layer to resume the RRC connection.

Editor's note:	it is FFS how to apply UAC during resume of the N1 NAS signalling connection.

-	Upon indication from the lower layers that the RRC connection has been resumed when in 5GMM-IDLE mode with suspend indication, the UE shall enter 5GMM-CONNECTED mode. If the pending NAS message is a SERVICE REQUEST message, the message shall not be sent. Otherwise the UE shall cipher the message as specified in subclause 4.4.5 and send the pending initial NAS message upon entering 5GMM-CONNECTED mode;

Editor's note:	it is FFS under which condition the UE shall not send the pending control plane service request message when the RRC connection has been resumed.

-	Upon indication from the lower layers that the RRC connection resume fails when in 5GMM-IDLE mode with suspend indication, the UE shall enter 5GMM-IDLE mode.

Editor's note:	it is FFS the further UE action when the RRC connection resume fails.

In the network, when user plane CIoT 5GS optimization is used:

-	Upon indication from the lower layers that the RRC connection has been suspended, the network shall enter 5GMM-IDLE mode with suspend indication, shall not consider the N1 NAS signalling connection released and shall not consider the secure exchange of NAS messages terminated; and

-	Upon indication from the lower layers that the RRC connection has been resumed when in 5GMM-IDLE mode with suspend indication, the network shall enter 5GMM-CONNECTED mode.

***** Next change *****