3GPP TSG-CT Meeting #85	CP-192249

# Newport Beach California, US, 16th Sep 2019 - 17th Sep 2019            was (CP-192031)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.510 | CR | 0226 | rev | 5 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Static IP Address |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Cisco Systems |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Cisco Systems |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-CT |  |  |  |  |  | Date: |  |  | 2019-09-04 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As agreed in SA2#134 CR S2-1907845, UPF while registering to NRF, will also provide UE IPv4 Address Ranges and/or IPv6 Prefix Range(s) per (S-NSSAI, DNN) so that SMFcan select the appropriate UPF based on the static IP address/prefix that is received in Session Management Subscription Data from UDM. Please note the below agreed excerpt from the CR “UPFs may be associated with UPF Provisioning Information in the NRF. The UPF Provisioning Information consists of: - a list of (S-NSSAI, DNN); - UE IPv4 Address Ranges and/or IPv6 Prefix Range(s) per (S-NSSAI, DNN); and NOTE 2: The above information can be used by the SMF for UPF selection when static IP address/prefix allocation is required for a UE. ” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of static IP address range in upfInfo, which is used during UPF registration with NRF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SMF cannot select the appropriate UPF based on the static IP address/prefix that is received in Session Management Subscriptoin Data from UDM. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.15, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS23.502 CR 1613. |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible corrections to the OpenAPI Nnrf_NFManagement and Nnrf_NFDiscovery as specified in TS 29.510. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev4 (CT#85): added “and Nnrf_NFDiscovery” in the Other comments section in cover page |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.1.6.2.15	Type: DnnUpfInfoItem

Table 6.1.6.2.15-1: Definition of type DnnUpfInfoItem

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| dnn |  | Dnn |  | M |  | 1 |  | Supported DNN |  |
| dnaiList |  | array(Dnai) |  | O |  | 1..N |  | List of Data network access identifiers supported by the UPF for this DNN. The absence of this attribute indicates that the UPF can be selected for this DNN for any DNAI. |  |
| pduSessionTypes |  | array(PduSessionType) |  | O |  | 1..N |  | List of PDU session type(s) supported by the UPF for a specific DNN. The absence of this attribute indicates that the UPF can be selected for this DNN for any PDU session type supported by the UPF (see clause 6.1.6.2.13). |  |
| ipv4AddressRanges |  | array(Ipv4AddressRange) |  | O |  | 1..N |  | List of ranges of static IPv4 addresses handled by UPF. |  |
| ipv6PrefixRanges |  | array(Ipv6PrefixRange) |  | O |  | 1..N |  | List of ranges of static IPv6 prefixes handled by the UPF. |  |

* * * Next Change * * * *

## A.2	Nnrf_NFManagement API

******** not shown for Clarity**********

DnnUpfInfoItem:

type: object

required:

- dnn

properties:

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

dnaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

pduSessionTypes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionType'

minItems: 1

ipv4AddressRanges:

type: array

items:

$ref: '#/components/schemas/Ipv4AddressRange'

minItems: 1

ipv6PrefixRanges:

type: array

items:

$ref: '#/components/schemas/Ipv6PrefixRange'

minItems: 1

******** not shown for Clarity**********

* * * End of Change * * * *