3GPP TSG-CT WG1 Meeting #119	C1-196815

# Wroclaw (Poland), 26-30 August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.379 | CR | 0501 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Automatic activation and deactivation of functional aliases based on location |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MONASTERY2 |  |  |  |  |  | Date: |  |  | 02/09/2019 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Stage 1 specs 3GPP TS 22.280 [R-5.9a-019] requires a mechanism to define a functional alias with related geographic areas that can be associated to MC service users for routing location dependent private communications. Stage 2 requirement (S6-190938) is re-using existing functional alias management procedures, and updates the MCPTT user profile data as specified in 3GPP TS 23.379. In stage 3, the addition of location criteria as triggers for FA activation/deactivation is needed and the necessary procedures are specified by the present CR |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Addition of location based functional alias activation procedure. 2) Add support of suppressing manual deactivation of FA if location criteria-for activation are met. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 1 & stage 2 requirements on automatic activation and deactivation of functional aliases based on location are not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9A.2.1.1, 9A.2.1.2, 9A.2.1.4(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9A.2.1.1	General

The MCPTT client procedures consist of:

-	a functional alias status change procedure;

-	a functional alias status determination procedure; and

-	a location based functional alias status change procedure.

In order to obtain information about success or rejection of changes triggered by the functional alias status change procedure for an MCPTT user, the MCPTT client needs to initiate the functional alias status determination procedure for the MCPTT user before starting the functional alias status change procedure for the MCPTT user.

* * * Second Change * * * *

#### 9A.2.1.2	Functional alias status change procedure

In order:

-	to indicate that an MCPTT user requests to activate one or more functional aliases;

-	to indicate that the MCPTT user requests to deactivate one or more functional aliases;

-	to refresh indication of an MCPTT user interest in one or more functional aliases due to near expiration of the expiration time of a functional alias with the status set to the "activated" state received in a SIP NOTIFY request in subclause 9A.2.1.3;

-	to indicate that the MCPTT client entering into or exiting from a location area triggers one or more functional aliases to be activated;

-	to indicate that the MCPTT client entering into or exiting from a location area triggers one or more functional aliases to be deactivated; or

-	any combination of the above;

the MCPTT client shall generate a SIP PUBLISH request according to 3GPP TS 24.229 [4], IETF RFC 3903 [37], and IETF RFC 3856 [51].

When the MCPTT user requests to deactivate a functional alias, the MCPTT client shall first check the <manual-deactivation-not-allowed-if-location-criteria-met> element within the <anyExt> element of the <entry> element corresponding to the functional alias within the <FunctionalAliasList> list element of the <anyExt> element of the <OnNetwork> element of the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.484 [50]). If the functional alias has been activated due to a location area trigger and the <manual-deactivation-not-allowed-if-location-criteria-met> element is set to a value of "true", the MCPTT client shall suppress the MCPTT user’s request.

NOTE 1:	If the request is suppressed, a notification message can be displayed to the user.

In the SIP PUBLISH request, the MCPTT client:

1)	shall set the Request-URI to the public service identity identifying the originating participating MCPTT function serving the MCPTT user;

2)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body. In the application/vnd.3gpp.mcptt-info+xml MIME body, the MCPTT client shall include the <mcptt-request-uri> element set to the MCPTT ID of the MCPTT user;

3)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Preferred-Service header field according to IETF RFC 6050 [9];

4)	if the MCPTT client requests to activate one or more functional aliases, shall set the Expires header field according to IETF RFC 3903 [37], to 4294967295;

NOTE 1:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [24].

5)	if the MCPTT client requests to deactivate one or more functional aliases, shall set the Expires header field according to IETF RFC 3903 [37], to zero; and

NOTE 2:	Activation and deactivation of functional alias cannot be performed with the same PUBLISH request.

6)	shall include an application/pidf+xml MIME body indicating per-user functional alias information according to subclause 9A.3.1. In the MIME body, the MCPTT client:

a)	shall include all functional aliases where the MCPTT user requests activation for the MCPTT ID;

b)	shall include the MCPTT client ID of the targeted MCPTT client;

c)	shall not include the "status" attribute and the "expires" attribute in the <functionalalias> element;

d)	if the MCPTT client has received an indication that take over of a functional alias is possible and intends to take over a functional alias, shall include a <take-over> child element set to "true"; and

e)	shall set the <p-id-fa> child element of the <presence> root element to a globally unique value.

The MCPTT client shall send the SIP PUBLISH request according to 3GPP TS 24.229 [4].

* * Third Change * * * *

#### 9A.2.1.4	Location based functional alias status change procedure

If a location criterion for functional alias activation or de-activation is met, the MCPTT client shall initiate the functional alias status change procedure as specified in subclause 9A.2.1.2.

* * * End of Change * * * *