3GPP TSG-CT Meeting #86	CP-193149

# Sitges, SPAIN; 09th – 10th December 2019

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	Video enhancement of IMS CAT/CRS/announcement services

## Acronym: eIMSVideo

## Unique identifier: 	860004

Potential target Release: Rel-16

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  |  |
| Don't know |  |  |  |  | X |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a Feature.

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) | Nature of relationship |

Dependency on non-3GPP (draft) specification:

None

## 3	Justification

According to current 3GPP specification, audio or video media may be provided to users as ring tone or announcement around IMS call.

Currently, some networks and UEs can support video well now. There are possible needs to clarify the conditions to provide video customized alerting/ringing tone and announcement. So that operators can get benefits by utilizing resources around audio/video call to provide video media for CAT/CRS and announcement. There are the cases for providing video CAT, video CRS and video announcement for IMS call. It is observed that some obstacles on the road of developing these services need to be removed by developing related standard specifications. For example:

1)	Current CAT/CRS specifications do not support providing video CAT/CRS to the UE which needs to use precondition mechanism for video CAT/CRS but the network disables precondition or terminating UE does not support the precondition.

2)	Current announcement specification does not specify the conditions for providing video announcement clearly, especially video announcement during established audio call, including:

a) the condition(e.g., the AS can only play video announcement to the UE which has video handling capability) for the AS to provide video announcement is not specified in TS 24.628;

b) TS 24.628 supports providing announcement during the established communication, but the method of transferring announcement media and the media for call separately at the same time during established communication is not specified.

Main impacts of this WID include:

1)	video CAT/CRS can be provided to more UEs by precondition mechanism;

2)	video announcement can be provided in a separate stream during the established audio communication;

3)  conditions for the AS to provide video announcement will be clear.

## 4	Objective

The objective of the work item is to enhance the video CAT service, video CRS service and video announcement service for IMS call without specifying the actions of IMS network element. The following areas of work are expected to be covered:

CT1:

1)	introduce the CAT negotiation function for CAT AS to provide video CAT to an originating UE which needs precondition mechanism for video CAT, when the basic call does not use precondition;

2)	introduce the CRS negotiation function for CRS AS to provide video CRS to an terminating UE which needs precondition mechanism for video CRS, when the basic call does not use precondition;

3)	clarify the conditions for providing video announcement, and consider transferring announcement media and the media for call separately at the same time during the established communication.

## 5	Expected Output and Time scale

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 24.182 | Introduce possible solution: the CAT negotiation function for providing video CAT to an originating UE which needs precondition mechanism for video CAT, when the basic call does not use precondition. | TSG#87 |  |
| TS 24.183 | Introduce possible solution: the CRS negotiation function for providing video CRS to a terminating UE which needs precondition mechanism for video CRS, when the basic call does not use precondition. | TSG#87 |  |
| TS 24.628 | Clarify the possible conditions for providing announcement on a media stream not existing in the session. | TSG#87 |  |

## 6	Work item Rapporteur(s)

Hongxia Hao, Huawei, haohongxia@huawei.com

## 7	Work item leadership

CT1

## 8	Aspects that involve other WGs

None

## 9	Supporting Individual Members

Supporting IM name

Huawei

HiSilicon

China Mobile

China Unicom

vivo

China Telecom

MediaTek Inc.