| 3GPP TS 24.535 V1.0.0 (2019-12) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Device-Side Time Sensitive Networking (TSN) Translator (DS-TT) to Network-Side TSN Translator (NW-TT) protocol aspects; Stage 3 (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	General	7

5	gPTP message delivery	7

5.1	Overview	7

5.2	Signalling of ingress time for time synchronization	7

5.3	Encoding of organization specific TLV extensions	7

5.3.1	General	7

5.3.2	Ingress timestamp	9

Annex A (informative): Change history	10

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, certain modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

NOTE 1:	The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

NOTE 2:	The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

NOTE 3:	The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

NOTE 4:	The constructions "can" and "cannot" shall not to be used as substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

NOTE 5:	The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the protocols of communication between a DS-TT and a NW-TT as specified in 3GPP TS 23.501 [2] for:

a)	gPTP message delivery.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	IEEE 1588-2008: "IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control".

[4]	IEEE Std 802.1AS-Rev/D7.3, August 2018: "IEEE Standard for Local and metropolitan area networks--Timing and Synchronization for Time-Sensitive Applications".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.501 [2] apply:

5G System

Time Sensitive Communication

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GS	5G System

DS-TT	Device-Side TSN Translator

gPTP	Generalized Precision Time Protocol

NW-TT	Network-Side TSN Translator

OUI	Organizational Unique Identifier

TSC	Time Sensitive Communication

TSN	Time-Sensitive Networking

# 4	General

For time sensitive communication (TSC), a 5G system (5GS) can be integrated as a bridge in a time-sensitive networking (TSN) network (i.e. a TSN bridge). The device-side TSN translator (DS-TT) is deployed at the UE-side edge and the network-side TSN translator (NW-TT) is deployed at the network-side edge in order to interface with a TSN network while achieving transparency (see 3GPP TS 23.501 [2]).

The DS-TT and NW-TT fulfil all functions related to IEEE 802.1AS [4], which include delivery of generalized precision time protocol (gPTP) messages. Clause 5 describes details of the gPTP message delivery.

# 5	gPTP message delivery

## 5.1	Overview

For TSN time synchronization, a 5G system (5GS) can be considered as a time-aware system, which needs to support implementation of generalized precision time protocol (gPTP) requirements (see IEEE 802.1AS [4]).

Within a 5GS, a gPTP message is delivered over the user plane. While the generic mechanisms for the gPTP message delivery are according to IEEE 802.1AS [4], a specific requirement for the DS-TT and the NW-TT is available in terms of the signalling of ingress time. See clause 5.2 for further details.

## 5.2	Signalling of ingress time for time synchronization

Upon reception of a downlink gPTP message (Sync message for one-step operation or Follow_up message for two-step operation) for a given TSN working domain, the NW-TT shall create an ingress timestamping (TSi) for each gPTP event (Sync) message and add TSi in the Suffix field of the gPTP message encoded as specified in clause 6.2.

Upon reception of a gPTP message over the user plane, the UE shall forward the gPTP message to the DS-TT. The DS-TT shall use TSi to calculate the residence time spent within the 5G system for the gPTP event (Sync) message expressed in 5GS time as specified in 3GPP TS 23.501 [2] for the corresponding TSN working domain, then shall remove TSi from the Suffix field of the gPTP message before sending the gPTP message toward the downstream TSN node.

In case of multiple TSN working domains, the procedure above is repeated for each TSN working domain.

## 5.3	Encoding of organization specific TLV extensions

### 5.3.1	General

Organization specific TLV extensions are included the Suffix field of a gPTP message as specified in clause 14.3 of IEEE 1588-2008 [3]. The Suffix field is coded as shown in figure 5.3.1.1 and table 5.3.1.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TLV type |  |  |  |  |  |  |  | octet 1 octet2 |
| Length of TLV |  |  |  |  |  |  |  | octet 3 octet 4 |
| Organization Id |  |  |  |  |  |  |  | octet 5 octet 7 |
| Organization Subtype |  |  |  |  |  |  |  | octet 8 octet 10 |
| Data field |  |  |  |  |  |  |  | octet 11 octet n |

Figure 5.3.1.1: Suffix field

Table 5.3.1.1: Suffix field

TLV type (octets 1 to 2)

This field indicates the type of TLV extension and shall be coded as follows: - 00003H ORGANIZATION_EXTENSION; All other values are reserved. Length of TLV (octets 3 to 4) This field indicates the length of the value part of the TLV extension (i.e. octets 5 to n) coded in binary over 2 octets. Organization Id (octets 5 to 7) This field indicates the value of the Organizational Unique Identifier (OUI) assigned to 3GPP by the IEEE, coded in binary over 3 octets. Organization Subtype (octets 8 to 10) This field identifies the type of TLV extension included in the Data field and shall be coded as follows: - 00000H Reserved; - 00001H Ingress timestamp

All other values are spare. Data field (octets 11 to n) This field contains the contents of the specific TLV extension. Its encoding is specified in the corresponding clause.

Editor's note: The value of the OUI for 3GPP needs to be assigned by the IEEE.

### 5.3.2	Ingress timestamp

The Data field of a TLV extension carrying an Ingress timestamp is coded as shown in figure 5.3.2.1 and table 5.3.2.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data field |  |  |  |  |  |  |  | octet 11 octet 20 |

Figure 5.3.2.1: Data field for Ingress time stamp (Organization Subtype = 0001H)

Table 5.3.2.1: Data field for Ingress time stamp (Organization Subtype = 0001H)

Data field (octets 11 to 20)

For Organization Subtype = 0001H, the data field contains the ingress timestamp of the gPTP event (Sync) message, encoded over 10 octets as specified in clause 5.3.3. of IEEE 1588-2008 [3].

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-08 | CT1#119 | C1-194647 |  |  |  | Draft skeleton provided by the rapporteur | 0.0.0 |
| 2019-09 | CT1#119 CT#85 | C1-194650 CP-192258 |  |  |  | Implementation of an agreed pseudo CR (C1-194650) Addition of the TS number assigned in CT#85 | 0.1.0 |
| 2019-11 | CT1#121 |  |  |  |  | Implementation of the following pseudo CRs agreed by CT1: C1-198476, C1-198749, C1-198750, and C1-199023 Corrections made by the rapporteur | 0.2.0 |
| 2019-12 | CT-86 | CP-193151 |  |  |  | Presentation for information to TSG CT | 1.0.0 |