3GPP TSG-CT Meeting #87e	CP-200266

# E-Meeting, 16th – 18th March 2020										(Revision of C3-201526)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.522 | CR | 0124 | rev | 3 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | OpenAPI file of Nnef_ServiceParameter Service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  | Date: |  |  | 2020-03-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Nnef_ServiceParameter service is defined in stage 2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define the OpenAPI file of the Nnef_ServiceParameter service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces a backwards compatible feature to the OpenAPI file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | C3-201526 is revised: Aligned with C4-201028, in OpenAPI file, Pc5QosPara is changed Pc5QoSPara; Pc5QoSFlowItem is changed to Pc5QosFlowItem. |  |  |  |  |  |  |  |  |

*** 1st Change ***

A.x	ServiceParameter API

openapi: 3.0.0

info:

title: 3gpp-service-parameter

version: 1.0.0.alpha-1

description: |

API for AF service paramter

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.522 V16.3.0; 5G System; Network Exposure Function Northbound APIs.

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.522/'

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/3gpp-service-parameter/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 5.2.4 of 3GPP TS 29.122.

paths:

/{afId}/subscriptions:

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

get:

summary: read all of the active subscriptions for the AF

tags:

- ServiceParameter API SCS/AS level GET Operation

responses:

'200':

description: OK.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/ServiceParameterData'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

post:

summary: Creates a new subscription resource

tags:

- ServiceParameter API Subscription level POST Operation

requestBody:

description: Request to create a new subscription resource

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceParameterData'

responses:

'201':

description: Created (Successful creation of subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceParameterData'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/{afId}/subscriptions/{subscriptionId}:

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

- name: subscriptionId

in: path

description: Identifier of the subscription resource

required: true

schema:

type: string

get:

summary: read an active subscriptions for the SCS/AS and the subscription Id

tags:

- ServiceParameter API Subscription level GET Operation

responses:

'200':

description: OK (Successful get the active subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceParameterData'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Updates/replaces an existing subscription resource

tags:

- TrafficInfluence API subscription level PUT Operation

requestBody:

description: Parameters to update/replace the existing subscription

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceParameterData'

responses:

'200':

description: OK (Successful update of the subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceParameterData'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Updates/replaces an existing subscription resource

tags:

- ServiceParameter API subscription level PATCH Operation

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/ServiceParameterDataPatch'

responses:

'200':

description: OK. The subscription was modified successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceParameterData'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes an already existing subscription

tags:

- ServiceParameter API Subscription level DELETE Operation

responses:

'204':

description: No Content (Successful deletion of the existing subscription)

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

ServiceParameterData:

type: object

properties:

afServiceId:

type: string

description: Identifies a service on behalf of which the AF is issuing the request.

appId:

type: string

description: Identifies an application.

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

externalGroupId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/ExternalGroupId'

anyUeInd:

type: boolean

description: Identifies whether the AF request applies to any UE. This attribute shall set to "true" if applicable for any UE, otherwise, set to "false".

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

ueIpv4:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv4Addr'

ueIpv6:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv6Addr'

ueMac:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MacAddr48'

self:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Link'

paramOverPc5:

$ref: '#/components/schemas/ParameterOverPc5'

paramOverUu:

$ref: '#/components/schemas/ParameterOverUu'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

ServiceParameterDataPatch:

type: object

properties:

paramOverPc5:

$ref: '#/components/schemas/ParameterOverPc5Rm'

ParamOverUu:

$ref: '#/components/schemas/ParameterOverUuRm'

ParameterOverPc5:

type: object

properties:

expiry:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

plmmRatServed:

type: array

items:

$ref: '#/components/schemas/PlmnRatServed'

minItems: 1

authNotServed:

type: boolean

radioParamsNotServed:

type: array

items:

$ref: '#/components/schemas/RadioParameterNotServed'

minItems: 1

serToTx:

type: array

items:

$ref: '#/components/schemas/ServiceToTx'

minItems: 1

privacyParams:

type: array

items:

$ref: '#/components/schemas/PrivacyParameter'

minItems: 1

configParaEutra:

$ref: '#/components/schemas/ConfigurationParametersEutra'

configParaNr:

$ref: '#/components/schemas/ConfigurationParametersNr'

ParameterOverUu:

type: object

properties:

expiry:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

serToPduSess:

type: array

items:

$ref: '#/components/schemas/ServiceToPduSession'

minItems: 1

serToAppAddr:

type: array

items:

$ref: '#/components/schemas/ServiceApplicationServerAddress'

minItems: 1

PlmnRatServed:

type: object

properties:

plmn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Plmn'

rats:

type: array

items:

$ref: '#/components/schemas/V2xRatType'

required:

- plmn

- rats

RadioParameterNotServed:

type: object

properties:

radioParams:

type: string

rat:

$ref: '#/components/schemas/V2xRatType'

geographicalArea:

type: string

operManaged:

$ref: '#/components/schemas/OperatorManaged'

required:

- radioParams

- geographicalArea

- operManaged

- rat

ServiceToTx:

type: object

properties:

serIds:

type: array

items:

type: string

minItems: 1

txProfile:

type: string

rat:

$ref: '#/components/schemas/V2xRatType'

required:

- serIds

- txProfile

- rat

PrivacyParameter:

type: object

properties:

serIds:

type: array

items:

type: string

minItems: 1

geographicalArea:

type: string

timer:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

required:

- serIds

- geographicalArea

- timer

ConfigurationParametersEutra:

type: object

properties:

serToLayer2Ids:

type: array

items:

$ref: '#/components/schemas/ServiceIdToLayer2Id'

minItems: 1

ppppToPdbs:

type: array

items:

$ref: '#/components/schemas/PpppToPdb'

minItems: 1

serIdToFrequs:

type: array

items:

$ref: '#/components/schemas/ServiceIdToFrequency'

minItems: 1

serIdToPpprs:

type: array

items:

$ref: '#/components/schemas/ServiceIdToPppr'

minItems: 1

ConfigurationParametersNr:

type: object

properties:

serToBroLayer2Ids:

type: array

items:

$ref: '#/components/schemas/ServiceIdToLayer2Id'

minItems: 1

serToGroLayer2Ids:

type: array

items:

$ref: '#/components/schemas/ServiceIdToLayer2Id'

minItems: 1

serToDefLayer2Ids:

type: array

items:

$ref: '#/components/schemas/ServiceIdToLayer2Id'

minItems: 1

serIdToFrequs:

type: array

items:

$ref: '#/components/schemas/ServiceIdToFrequency'

minItems: 1

pc5QosMappings:

type: array

items:

$ref: '#/components/schemas/Pc5QosMapping'

minItems: 1

slrbConfigs:

type: array

items:

$ref: '#/components/schemas/SlrbConfigurations'

minItems: 1

ServiceIdToLayer2Id:

type: object

properties:

serIds:

type: array

items:

type: string

minItems: 1

desLayer2Id:

type: string

required:

- serIds

- desLayer2Id

PpppToPdb:

type: object

properties:

pppp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

pdb:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketDelBudget'

required:

- pppp

- pdb

ServiceIdToFrequency:

type: object

properties:

serIds:

type: array

items:

type: string

minItems: 1

frequency:

type: number

geographicalArea:

type: string

required:

- serIds

- frequency

- geopraphicalArea

ServiceIdToPppr:

type: object

properties:

serIds:

type: array

items:

type: string

minItems: 1

pppr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

required:

- serIds

- pppr

Pc5QosMapping:

type: object

properties:

serIds:

type: array

items:

type: string

minItems: 1

pc5QoSFlowParam:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pc5QosFlowItem'

required:

- serIds

- pc5QosFlowParam

SlrbConfigurations:

type: object

properties:

pc5QosPara:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pc5QoSPara'

slrb:

type: string

required:

- pc5QosPara

- slrb

ServiceToPduSession:

type: object

properties:

serIds:

type: array

items:

type: string

minItems: 1

pduSessionType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionType'

sscMode:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SccMode'

sliceInfo:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

dnns:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

minItems: 1

required:

- serIds

ServiceApplicationServerAddress:

type: object

properties:

serIds:

type: array

items:

type: string

minItems: 1

fqdn:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/Fqdn'

Ipv4Addrs:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

minItems: 1

Ipv6Addrs:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

minItems: 1

udpPortNumbers:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

minItems: 1

tcpPortNumbers:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

minItems: 1

plmns:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Plmn'

geopraphicalArea:

type: string

required:

- serIds

- plmns

- geographicalArea

ParameterOverPc5Rm:

type: string

# Editor's note: Type is FFS. String only used to pass Syntax check.

ParameterOverUuRm:

type: string

# Editor's note: Type is FFS. String only used to pass Syntax check.

OperatorManaged:

anyOf:

- type: string

enum:

- OPERATOR_MANAGED

- NON_OPERATOR_MANAGED

- type: string

description: >

Possible values are

- OPERATOR_MANAGED: The radio parameters are "operator managed".

- NON_OPERATOR_MANAGED: The radio parameters are "non-operator managed".

V2xRatType:

anyOf:

- type: string

enum:

- PC5_LTE

- PC5_NR

- type: string

description: >

Possible values are

- PC5_LTE: Indicates the LTE RAT type over PC5 interface..

- PC5_NR: Indicates the NR RAT type over NR interface.

*** End of Change ***