3GPP TSG-CT WG1 Meeting #124-e	C1-203793

Electronic meeting, 2-10 June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.379 | CR | 0618 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Schema error – FA Coding 9A.3.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | FirstNet |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MONASTERY |  |  |  |  |  | Date: |  |  | 4 June 2020 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The schema for the application/pidf+xml MIME body defined in subclause 9A.3.1.2 has an error in a type declaration. Existing text: ="mcpttPIFA10:fucntionalAliasType"/> needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed ="mcpttPIFA10:fucntionalAliasType"/> to ="mcpttPIFA10:functionalAliasType"/> |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect schema definition. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9A.3.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * FIRST CHANGE * * * * *

#### 9A.3.1.2	Syntax

The application/pidf+xml MIME body indicating per-user functional alias information is constructed according to IETF RFC 3863 [52] and:

1)	contains a <presence> root element according to IETF RFC 3863 [52];

2)	contains an "entity" attribute of the <presence> element set to the MCPTT ID of the MCPTT user;

3)	contains one <tuple> child element according to IETF RFC 3863 [52] per <presence> element;

4)	can contain a <p-id-fa> child element defined in the XML schema defined in table 9A.3.1.2-1, of the <presence> element set to an identifier of a SIP PUBLISH request;

5)	contains an "id" attribute of the <tuple> element set to the MCPTT user ID;

6)	contains one <status> child element of each <tuple> element;

7)	contains one <functionalAlias> child element defined in the XML schema defined in table 9A.3.1.2-1, of the <status> element, for each functional alias in which the MCPTT user is interested;

8)	contains a "functionalAliasID" attribute of each <functionalAlias> element set to the functional alias ID of the functional alias in which the MCPTT user is interested;

9)	can contain a "status" attribute of each <functionalAliasID> element indicating the activation status of functional alias for the MCPTT user; and

10)	can contain an "expires" attribute of each <functionalAlias> element indicating expiration of activation of the functional alias for the MCPTT user.

The application/pidf+xml MIME body indicating per-functional alias status information is constructed according to IETF RFC 3856 [51] and:

1)	contains the <presence> root element according to IETF RFC 3863 [52];

2)	contains an "entity" attribute of the <presence> element set to the functional alias ID of the functional alias;

3)	contains one <tuple> child element according to IETF RFC 3863 [52] of the <presence> element;

4)	can contain a <p-id-fa> child element defined in the XML schema defined in table 9A.3.1.2-1, of the <presence> element set to an identifier of a SIP PUBLISH request;

5)	contains an "id" attribute of the <tuple> element set to the functional alias ID;

6)	contains one <status> child element of each <tuple> element;

7)	contains one <functionalAlias> child element defined in the XML schema defined in table 9A.3.1.2-1, of the <status> element, for each MCPTT ID for which functional alias information is provided;

8)	contains one "user" attribute defined in the XML schema defined in table 9A.3.1.2-2, of the <functionalAlias> element set to the MCPTT user ID; and

9)	can contain an "expires" attribute defined in the XML schema defined in table 9A.3.1.2-2, of the <functionalAlias> element indicating expiration of activation of the functional alias for the MCPTT user.

Table 9A.3.1.2-1: XML schema with elements and attributes extending the application/pidf+xml MIME body

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema

targetNamespace="urn:3gpp:ns:mcpttPresInfoFA:1.0"

xmlns:xs="http://www.w3.org/2001/XMLSchema"

xmlns:mcpttPI10="urn:3gpp:ns:mcpttPresInfoFA:1.0"

elementFormDefault="qualified" attributeFormDefault="unqualified">

<!-- MCPTT functional alias specific child elements of tuple element -->

<xs:element name="functionalAlias" type="mcpttPIFA10:functionalAliasType"/>

<xs:complexType name="functionalAliasType">

<xs:sequence>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="functionalAliasID" type="xs:anyURI" use="optional"/>

<xs:attribute name="user" type="xs:anyURI" use="optional"/>

<xs:attribute name="status" type="mcpttPIFA10:statusType" use="optional"/>

<xs:attribute name="expires" type="xs:dateTime" use="optional"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="statusType">

<xs:restriction base="xs:string">

<xs:enumeration value="activating"/>

<xs:enumeration value="activated"/>

<xs:enumeration value="deactivating"/>

<xs:enumeration value="take-over-possible"/>

</xs:restriction>

</xs:simpleType>

<xs:element name="p-id-fa" type="xs:string"/>

<xs:element name="take-over" type="xs:boolean"/>

</xs:schema>

The application/pidf+xml MIME body refers to namespaces using prefixes specified in table 9A.3.1.2-2.

Table 9A.3.1.2-2: Assignment of prefixes to namespace names in the application/pidf+xml MIME body

| Prefix | Namespace |
| --- | --- |
| mcpttPIFA10 | urn:3gpp:ns:mcpttPresInfoFA:1.0 |
| NOTE: The "urn:ietf:params:xml:ns:pidf" namespace is the default namespace so no prefix is used for it in the application/pidf+xml MIME body. |  |

* * * * * END CHANGES * * * * *